| 3GPP TR 23.700-10 V0.2.0 (2020-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Study on enhanced IP Multimedia Subsystem (IMS) to 5GC integration Phase 2; CT WG1 aspects (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	6

4	Architecture Assumptions	7

5	Key Issues	7

5.1	Key issue 1: Network Slicing and IMS	7

5.2	Key issue 2: Routing of IMS traffic via a localized UPF	9

5.2.1	Description	9

5.2.2	Requirements	9

5.3	Key issue 3: Placement of IMS application server in localized environments	10

5.3.1	Description	10

5.3.2	Requirements	10

5.4	Key issue #4: IMS utilizing services provided by 5GC NFs other than PCF and HSS	11

5.x	Key issue #X: <Key Issue Title>	11

5.x.1	Description	11

5.x.2	Requirements	11

6	Solutions	11

6.1	Solution 1: 5GC network slice selection for IMS traffic route via dedicated slice(s)	11

6.1.1	Description	11

6.1.1.1	General	11

6.1.1.2	Procedure	12

6.1.2	Impacts on existing nodes and functions	12

6.1.3	Solution Evaluation	12

6.x	Solution #X: <Solution Title>	13

6.x.1	Description	13

6.x.2	Impacts on existing nodes and functions	13

6.x.3	Solution Evaluation	13

7	Overall Evaluation	13

8	Conclusions	13

Annex <A> (informative): Change history	14

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The aim of this Technical Report is to study and specify potential enhancements to the IMS architecture to enable IMS FE to integrate with the 5GC network functions to enable IMS applications to directly leverage the features and capabilities of 5GC.

The specific capabilities of 5GC included in this study:

-	In regard to 5GC network slicing; what enhancements (if any) are necessary for IMS and IP-CAN aspects of the IMS to efficiently support devices and networks with multiple slices and different services offered by the IMS network per slice?

Editor's note:	Additional scope needs to be studied.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.502: "Procedures for the 5G System".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

3.2	Symbols

For the purposes of the present document, the following symbols apply:

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

NSSAI	Network Slice Selection Assistance Information

S-NSSAI	Single NSSAI

URSP	UE Route Selection Policy

4	Architecture Assumptions

5	Key Issues

5.1	Key issue 1: Network Slicing and IMS

In release 16, 5GC developed capabilities to divide the network into slices to better offer services to users and devices - in many cases the details of network slicing do not impact IMS, however if the IMS domain offers different services to different 5GC network slices there may be some impacts. This key issue investigates the impacts to the IMS domain due to 5GC network slicing, and how a IMS network can ensure a user or device with multiple 5GC network slices and multiple services offered by IMS from utilizing the appropriate 5GC network slice for each service on IMS and prevent inappropriate services been used from the incorrect 5GC network slice.

Examples of the use cases that solutions to this key issue will enable include:

Use Case A: Different 5GC slices optimized for different services on IMS

In this use case each 5GC slice supports a different services offered by IMS and the 5GC slice is optimized for the media type associated with the service on IMS. It is possible that either a shared or separate IMS network provides the services for each slice (scenario 1 or scenario 2 described below).

Figure 5.1.1 shows an example implementation of this use case where 3 5GC slices are used, one for IMS service #1, one for IMS service #2, and one for IMS service #3. The diagram shows the association of the services on IMS to their appropriate slice, the services on IMS may be supported by a shared IMS network (i.e. Scenario 2), in which case the 5GC slices will connect to through the common IMS network.

Figure 5.1.1: Example use case with different 5GC slices optimized for different services on IMS

Editor's note:	The definition of IMS service in the figure is FFS.

Use Case B: Different 3rd party 5GC slices with support for 3rd party services on IMS

In this use case separate 5GC slices are operated by different parties (some separate from the network operator); each 5GC slice is used to support services on IMS according to the 5GC slice operator's service needs; these services on IMS may be different or duplicated per slice according to service requirements. Additionally, some 5GC slices may provide non-IMS services. While it is possible that a shared IMS network provides the services for each slice (as per scenario 2 described below), this is an unlikely implementation and it is assumed each party has their own IMS network (as per scenario 1 described below).

Figure 5.1.2 shows an example implementation of this use case where 3 5GC slices are provided, one provided by an automobile manufacturer, one provided by an automotive entertainment service provider, and one provided by a network operator. A different mix of IMS and connectivity services are provided by each 5GC slice. The diagram shows the association of the services on IMS to their appropriate slice, however as each slice is provided by different parties it is unlikely that the services on IMS share an IMS network (as in Scenario 2); thus each 5GC slice would be associated with a different IMS network supporting the services on IMS associated with the 5GC slice.

Figure 5.1.2: Example use case with different 3rd party 5GC slices providing services on IMS

Editor's note:	The definition of IMS service in the figure is FFS.

Two scenarios are possible (and may be mixed when more than 2 slices are considered):

Scenario 1 - Each 5GC network slice is associated with a separate and distinct IMS network

Figure 5.1-3 UE connects to two IMS networks through two 5GC network slices

Scenario 2 - Each 5GC slice is associated with a common IMS network

Figure 5.1-4: UE connects to common IMS network through two 5GC network slices

Solutions may relate to one, or both, of these scenarios.

The following is a partial list of what aspects the solutions to this key issue need to address:

-	What information the UE needs to be able to select an appropriate slice.

-	How the IMS network selects an appropriate slice.

-	How the IMS network handles unavailability of a slice.

-	When the IMS FEs and UE are in different PLMNs (i.e. roaming), how the two networks interact.

-	Whether the service offered by the IMS network is aware of the 5GC slice information.

-	If additional information is needed in the IMS signalling between the IMS network and the UE

-	Any changes to the P-CSCF discovery procedures due to 5GC slicing.

-	If any additional information is needed for the UE and IMS network to identify services supported by the IMS network.

-	IMS backward compatibility to Rel-16.

-	How an appropriate slice for a user initiating or upgrading to Multimedia Priority Service (MPS) is selected.

Editor's note:	It is FFS which scenario each aspect relates to.

5.2	Key issue 2: Routing of IMS traffic via a localized UPF

5.2.1	Description

The 5GC enables traffic to route via localized UPF close to the edge of the network (in some cases adjacent to the RAN nodes). Some services offered by the IMS network may be able to benefit from the lower latency and/or lower backhaul requirements that such deployments can enable, however current IMS applications and services are not able to leverage these capabilities. This key issue investigates the interactions required to leverage localized routing of IMS media and signalling traffic, the changes to enable effective control and management of such routing, and impacts of mobility and roaming.

5.2.2	Requirements

Example Use Case A:

Routing of IMS traffic (e.g. video) between two (or more) users using an efficient UP path.

Example Use Case B:

A "best effort video service" where the IMS video traffic from UE is offloaded at a UPF closer to the UE to minimize the backhaul resource usage based on a decision by the IMS.

The following is a partial list of the issues to be addressed:

-	How can the IMS network influence or control how the 5GC selects the UP path.

-	Which IMS node interacts with the 5GC (i.e. IMS AS, S-CSCF, or other IMS node).

-	How does an AS request traffic to route using a local UPF.

-	How does IMS request traffic to route using (or prevent routing using) a local UPF.

-	How is the mobility of the UE considered, and how is service continuity ensured.

-	If the UE is roaming, how can the localized routing be extended into the VPLMN (if supported).

-	How does IMS request Data channel media to route using (or prevent using) a local UPF.

5.3	Key issue 3: Placement of IMS application server in localized environments

5.3.1	Description

The 5GC in release 15 defined some functionality to enable "mobile edge computing" (i.e. computing resources located close to the edge of the network) to enable enhanced services to users. The functionality defined in release 15 focused on general compute applications, and did not analyse the special case of IMS applications. This key issue investigates the enhancements to current capabilities of IMS and 5GC to enable the operation of IMS servers in the mobile edge space.

Editor's note:	It is FFS which functionality defined in release 16 should also be considered.

5.3.2	Requirements

Example Use Case A:

For a given venue (e.g. stadium) IMS videos may be stored and retrieved locally for playback only on UE's that are located on a group of small cells that serve the venue.

Example Use Case B:

Mission critical services that require optimization of signalling traffic as well as media may require, placement of P-CSCF and AGW (or other IMS FE) close to AN.

The following is a partial list of the issues to be addressed:

-	How can IMS leverage 5G support for edge computing.

-	What additions to IMS and 5G support for edge computing capabilities are required.

-	What restrictions are there (if any) on IMS applications in the edge network.

-	How are the localized IMS applications identified.

-	How are IMS applications outside the local PLMN impacted (e.g. roaming).

-	What impacts does the solution have on mobility.

-	What impacts does the solution have on service, and application continuity.

-	Is there any further impact on routing of IMS media (including Data channel media) and IMS signalling.

5.4	Key issue #4: IMS utilizing services provided by 5GC NFs other than PCF and HSS

All of the 5GC control plane NFs are specified as service based and any NF can be consumer of the services provided by other NFs, if supported by the procedures. In 3GPP Rel-16, it has been already defined how the IMS can utilize services provided by the PCF and HSS via Npcf and Nhss_ims interfaces, respectively.

In IMS network, the P-CSCF needs to be notified of some events related to access network. This information can be used in IMS network e.g., for emergency services, billing, LI, service trigger, etc.

An example of such an event related to the access network is the retrieval of the UE location information. For the purpose of UE location information retrieval in VoLTE case, the P-CSCF subscribes to notification of such events over Rx interface. Since 5GC supports SBI, IMS network can use directly the SBI provided by AMF to subscribe to notification of these events.

This key issue addresses the following aspects:

-	Which services provided by which 5GC NFs other than PCF and HSS are needed to be utilized by IMS.

-	Whether and how IMS can utilize services provided by 5GC NFs other than PCF and HSS.

-	For UE location information retrieval, how IMS can determine the UE location entities with service operations (e.g. determination of the visited PLMN ID in case of Home-Routed VoLTE roaming).

5.x	Key issue #X: <Key Issue Title>

5.x.1	Description

5.x.2	Requirements

6	Solutions

6.1	Solution 1: 5GC network slice selection for IMS traffic route via dedicated slice(s)

6.1.1	Description

6.1.1.1	General

This solution addresses scenario 2 of key issue#1 (Network Slicing and IMS) to ensure IMS service uses specific slice(s) in the 5GC.

The solution describes how the UE selects S-NSSAI(s) for IMS service including SIP signalling and media.

It is proposed that all IMS signalling related information is conveyed to the IMS network via a dedicated 5GC network slice and that media associated to a particular IMS session can be routed via different 5G network slice(s) based on the application's requirements.

When establishing a 5GS PDU session for IMS service, the UE selects an S-NSSAI from list of S-NSSAIs included in Allowed NSSAI based on URSP rules.

A URSP rule can e.g. contain an S-NSSAI for IMS service with associated registration area. If the current registration area associated to an S-NSSAI in Allowed NSSAI is available in the registration area associated to the S-NSSAI for IMS service in URSP rules, then UE selects the S-NSSAI for IMS service at current registration area. At IMS session setup, URSP rule(s) can also be used to establish dedicated 5GS PDU session(s) over the same or new S-NSSAI(s).

6.1.1.2	Procedure

The following flow describes the procedure for the UE selecting an S-NSSAI for IMS service.

Figure 6.1.1.2: Procedure to select an S-NSSAI for IMS service

The steps required are as follows:

1.	The UE performs registration procedure as described in clause 4.2.2 of 3GPP TS 23.502 [2]. Here, the registration accept message includes for example S-NSSAI #1.

2.	The AMF and PCF performs UE Policy Association Establishment as described in clause 4.16.11 of 3GPP TS 23.502 [2].

3.	Along with step 2, the PCF performs UE Configuration Update procedure as described in clause 4.2.4.3 of 3GPP TS 23.502 [2]. The MANAGE UE POLICY COMMAND message contains a URSP and the URSP contains for example S-NSSAI #1 for IMS service with available registration area and S-NSSAI #2 for other traffic.

4.	When UE requests to establish a 5GS PDU session for IMS service, if the UE determines S-NSSAI #1 in Allowed NSSAI is usable in the current registration area based on the URSP in which S-NSSAI #1 is associated to available registration area, then the UE selects S-NSSAI #1 for IMS service.

6.1.2	Impacts on existing nodes and functions

Editor's note:	This clause describes impacts to existing entities and interfaces.

6.1.3	Solution Evaluation

Editor's note:	This clause provides an evaluation of the solution.

6.x	Solution #X: <Solution Title>

6.x.1	Description

6.x.2	Impacts on existing nodes and functions

6.x.3	Solution Evaluation

7	Overall Evaluation

8	Conclusions

Annex <A> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-09 | CT1#125e | C1-205277 | - | - | - | Draft Skeleton | 0.0.0 |
| 2020-10 | CT1#126e | C1-206527 C1-206530 C1-206710 C1-206739 C1-206740 |  |  |  | Agreed pCRs in CT WG1#126e | 0.1.0 |
| 2020-11 | CT1#127e | C1-207706 |  |  |  | Agreed pCR in CT WG1#127e Corrupted Figure 6.1.1.2 fixed using Visio format | 0.2.0 |