| 3GPP TR 23.700-19 V0.2.0 (2025-05) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Integration of satellite components in the 5G architecture; Phase 4 (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	9

3.1	Terms	9

3.2	Symbols	9

3.3	Abbreviations	9

4	Architectural Assumptions	9

5	Key Issues	10

5.1	Key Issue #1: Support of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC	10

5.2	Key Issue #2: IMS enhancement for GEO NB-IoT NTN access	10

5.3	Key Issue #3: Support of IMS emergency call over NB-IoT NTN via GEO satellite connecting to EPC	10

5.5	Key Issue #5: UE-SAT-UE communication via UPF only onboard satellite for non-IMS services	11

6	Solutions	11

6.0	Mapping of Solutions to Key Issues	11

6.X	Solution #X: <Solution Title>	12

6.X.0	High-level solution Principles	12

6.X.1	Description	12

6.X.2	Procedures	12

6.X.3	Impacts on Services, Entities and Interfaces	12

6.1	Solution #1: UP based IMS voice call over NB-IoT NTN via GEO satellite using a default EPS bearer for both IMS signalling and voice data	13

6.1.0	High-level solution Principles	13

6.1.1	Description	13

6.1.2	Procedures	13

6.1.2.1 Attach or TAU procedure negotiating Voice call support over NB-IoT(Geo)(GEO):	13

6.1.2.2	Default EPS bearer for both IMS signalling and voice data	13

6.2	Solution #2: Supporting establishing EPS bearers for IMS voice via user plane with IP type PDN connection	14

6.2.0	High-level solution Principles	14

6.2.1	Description	14

6.2.2	Procedures	15

6.2.2.1	Attach and IMS PDN connection establishment	15

6.2.2.2	MO call	18

6.2.2.3	MT call	19

6.2.3	Impacts on Services, Entities and Interfaces	20

6.3	Solution #3: IP Type IMS PDN connection over User Plane with NIDD voice transfer between UE and PGW.	20

6.3.0	High-level solution Principles	20

6.3.1	Description	21

6.3.2	Procedures	21

6.3.2.1	Establishment and Usage of IP Type IMS PDN connection over User Plane with NIDD voice transfer between UE and PGW.	21

6.3.2.2	NIDD voice transfer between UE and PGW	22

6.3.2.2.1	Uplink Voice Data Transmission using NIDD over dedicated EPS bearer	22

6.3.2.2.1.1	UE side consideration	22

6.3.2.2.1.2	Network side consideration	22

6.3.2.2.1	Downlink Voice Data Transmission over NIDD over dedicated EPS bearer	22

6.3.3	Impacts on Services, Entities and Interfaces	22

6.4	Solution #4: Support of IMS call over NB-IoT NTN via NIDD and I1 protocol	23

6.4.0	High-level solution Principles	23

6.4.1	Description	24

6.4.1.1 	General	24

6.4.1.2	DRBs for signalling and media	24

6.4.1.3	Use of SCC-AS architecture for IMS Session Setup	24

6.4.1.4 	QoS support	26

6.4.2	Procedures	26

6.4.2.1 	MO Call Procedures	26

6.4.2.2 	MT Call Procedures	28

6.4.2.3 	Emergency Call Procedures	30

6.4.3	Impacts on Services, Entities and Interfaces	30

6.5	Solution #5: Support of IMS voice NB-IoT NTN connected to EPC using control plane bearers and new SRBs	31

6.5.0	High-level solution Principles	31

6.5.1	Description	31

6.5.1.1	Requirements	31

6.5.1.2	Network architecture	31

6.5.1.3	PDN connection types	31

6.5.1.4	Bearer mapping	32

6.5.1.4.1	Bearer mapping (General)	32

6.5.1.4.2	Bearer mapping when using IP type PDN connection	32

6.5.1.4.3	Bearer mapping when using non-IP type PDN connection	33

6.5.1.5	Protocol overhead for the voice packet flow	33

6.5.1.6	Protocol overhead reduction for the voice packet flow	35

6.5.1.6.1	RRC overhead reduction	35

6.5.1.6.2	NAS layer overhead reduction	35

6.5.2	Procedures	35

6.5.3	Impacts on Services, Entities and Interfaces	35

6.6	Solution #6: Support establishing dedicated bearer for QoS differentiation for CP based IMS voice service over NB-IoT (GEO)	36

6.6.0	High-level solution Principles	36

6.6.1	Description	36

6.6.2	Procedures	37

6.6.3	Impacts on Services, Entities and Interfaces	40

6.7	Solution #7: CP based IMS voice call over NB-IoT NTN via GEO satellite using an SRB for both IMS signalling and voice data	40

6.7.0	High-level solution Principles	40

6.7.1	Description	41

6.7.2	Procedures	41

6.7.2.1 Attach or TAU procedure negotiating Voice call support over NB-IoT(GEO):	41

6.7.2.2	Default EPS bearer for both IMS signalling and voice data	41

6.7.2.3	S1-U data transfer activation	42

6.8	Solution #8: Combined CP/UP bearers in a PDN connection	42

6.8.0	High-level solution Principles	42

6.8.1	Description	42

6.8.2	Procedures	43

6.8.2.1	Network architecture	43

6.8.2.2 Combined CP/UP bearers establishment	44

6.8.3	Impacts to Services, Entities and Interfaces	45

6.10	Solution #10: Support of CP/UP combination and QoS for IMS voice service over GEO satellite with a pair of PDN connections	49

6.10.0	High-level solution Principles	49

6.10.1	Description	49

6.10.2	Procedures	50

6.10.2.1	General	50

6.10.2.2	Establishment of PDN connection for SIP message exchange	50

6.10.2.3	IMS voice call setup	51

6.10.2.3.1	MO procedure	51

6.10.2.3.2	MT procedure	52

6.10.3	Impacts on Services, Entities and Interfaces	53

6.11	Solution #11: Support of PDN connection where packets are transported as Non-IP over the GEO satellite and seen as IP by IMS nodes	54

6.11.0	High-level solution Principles	54

6.11.1	Description	54

6.11.2	Procedures	55

6.11.2.1	General	55

6.11.2.2	Establishment of PDN connection for SIP message exchange	55

6.11.2.3	IMS registration	56

6.11.2.4	IMS voice call setup	57

6.11.2.4.1	MO procedure	57

6.11.2.4.2	MT procedure	60

6.11.3	Impacts on Services, Entities and Interfaces	61

6.12	Solution #12: Reduce SIP message size and number during INVITE procedure, P-CSCF ascts as B2BUA	62

6.12.2.1	Registration	63

6.12.2.2	Mobile originating	63

6.12.2.3	Mobile terminating	66

6.12.2.4	Information for call setup time evaluation	69

7	Interim Agreements	70

7.1	Agreed Principles for KI#5 of UE-SAT-UE communication via UPF only onboard satellite for non-IMS services	70

8	Conclusions	71

Annex A: Change history	71

Annex B: Methodology for estimating call setup time	72

B.1	Formula for estimated call setup time calculation	72

B.1.1	General	72

B.1.2	Estimated time of IMS procedure for call setup	72

B.1.3	Estimated time of EPS procedure during call setup	72

B.1.4	Estimated time of state transition	73

B.2	Example message size and corresponding transmission delay	73

Annex C: Mandatory parameters for SIP and SDP	74

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The scope of this Technical Report is to study the following aspects:

-	Support of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC.

-	Enhancements for UE-SAT-UE communication with 5GS under NR NTN NGSO constellation with feeder link available.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[6]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[7]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[8]	3GPP TS 22.261: "Service requirements for the 5G system".

[9]	3GPP TS 22.101: "Service aspects; Service principles".

[10]	3GPP TS 29.211: "Rx Interface and Rx/Gx signalling flows".

[11]	3GPP TS 29.213: "Policy and charging control signalling flows and Quality of Service (QoS) parameter mapping".

[12]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[13]	3GPP TS 24.008: "Mobile Radio Interface Layer 3 specification; Core Network Protocols; Stage 3".

[14]	3GPP TS 24.294: "IP Multimedia Subsystem (IMS) Centralized Services (ICS) protocol via I1 interface".

[15]	3GPP TS 23.292: "IP Multimedia Subsystem (IMS) centralized services; Stage 2".

[16]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[17]	3GPP TS 23.203: " Policy and charging control architecture".

[18]	RFC 3261: "SIP: Session Initiation Protocol".

[19]	RFC 4566: "SDP: Session Description Protocol".

[20]	RFC 5049: "Applying Signaling Compression (SigComp) to the Session Initiation Protocol (SIP)"

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Serving satellite: a satellite providing the satellite access to a UE (e.g. providing the serving cell(s)). Depending on the orbit, the serving satellite is covering a given geographic area for a limited period of time.

UE-Satellite-UE Communication: refers to a communication between UEs under the coverage of one or more serving satellites, using satellite access without the user traffic transiting through the ground segment.

NOTE:	These definitions are different from those in TS 22.261 [8].

NTN Gateway: an earth station located at the surface of the earth, providing connectivity to the NTN payload using the feeder link. An NTN Gateway is a TNL node.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

ISL	Inter-Satellite Link

NTN	Non-Terrestrial Network

# 4	Architectural Assumptions

For IMS voice call over NB-IoT (GEO), the following architecture assumptions and principles are applied to this study:

-	The EPC architecture with GEO satellite access for NB-IoT, as defined in TS 23.401 [5] is used as baseline.

NOTE 1:	CP CIoT EPS optimization, UP CIoT EPS optimization and S1-U based NB IoT NTN solutions will be studied.

NOTE 2:	Solutions should take into account that CP CIoT EPS optimization data in current pre Rel-20 NB-IoT is regarded as NAS signalling and by default has higher priority than UP data.

-	There will be no mobility enhancements developed for the NB-IoT NTN in this study.

-	Enhancements to NB-IoT (GEO), EPC, and IMS for supporting voice call via GEO shall be minimized.

-	Only architecture options with data via SGi will be considered in the study. The option with data via SCEF/T8 will not be considered.

-	The IMS reference architecture, as specified in TS 23.228 [6] for non-emergency calls is used as baseline.

For emergency IMS sessions over NB-IoT (GEO), the following additional architecture assumptions and principles are applied to this study, compared to above:

-	The IMS reference architecture, as specified in TS 23.167 [7] for emergency calls is the baseline.

-	Enhancements to LCS architecture for IMS emergency sessions and other regulatory services are out of scope of this study.

The following architecture assumptions are applied to the study for UE-Satellite-UE Communication:

-	The 5GS architecture for NR satellite access as defined in TS 23.501 [2] is used as a baseline.

-	Inter-Satellite Links (ISL) and Feeder link are assumed to act only as transport layer links and are not specified in 3GPP, and the use of ISL is assumed to have no impact on the reference points between core network entities when these core network entities are onboard.

-	NR UE-Satellite-UE Communication assumes a feeder link is always available.

-	NR UE-Satellite-UE Communication for non-IMS assumes a gNB and a UPF is present onboard of the satellite.

-	NR UE-Satellite-UE Communication assumes that the serving satellite will be of NSGO type.

-	NR UE-Satellite-UE Communication service assumes communication between UEs under the coverage of the same satellite or different satellites of the same PLMN.

-	No UE impacts are expected for UE-Satellite-UE Communications.

# 5	Key Issues

## 5.1	Key Issue #1: Support of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC

To support IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC, this key issue studies the following issues:

-	System enhancements to support IMS voice service over NB-IoT NTN via GEO satellite connecting to EPC;

-	Whether and how to support QoS for IMS voice service over GEO satellite.

## 5.2	Key Issue #2: IMS enhancement for GEO NB-IoT NTN access

To meet the requirements on experienced data rate and call setup time documented in TS 22.261 [8], this key issue studies:

-	Whether and how to enhance IMS to utilize NB-IoT as IP-CAN;

-	Potential IMS optimization.

## 5.3	Key Issue #3: Support of IMS emergency call over NB-IoT NTN via GEO satellite connecting to EPC

This key issue studies:

-	How to support IMS emergency call over NB-IoT NTN via GEO satellite connecting to EPC to comply with the relevant requirements defined in TS 22.261 [8] and TS 22.101 [9].

## 5.4	Key Issue #4: Location service for IMS emergency call and regulatory services over NB-IoT NTN

This key issue studies whether existing location services can be used to obtain the UE location (e.g., geographical location) meeting regulatory requirements, when the UE establishes an IMS emergency call over NB-IoT NTN via GEO satellite connecting to EPC or other services for which regulatory requirements for obtaining network provided location information exist.

## 5.5	Key Issue #5: UE-SAT-UE communication via UPF only onboard satellite for non-IMS services

This key issue studies:

-	How to support UE-SAT-UE communication via UPF only onboard satellite for non-IMS services (e.g. 5G-LAN services with IP and Ethernet PDU Session type).

NOTE 1:	Whether existing mechanisms can be reused will be considered in this KI.

NOTE 2:	Any UE impact will not be considered in this KI

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 |
| #1 | x | x |  |  |
| #2 | x |  |  |  |
| #3 | x |  |  |  |
| #4 | x | x | x | x |
| #5 | x |  |  |  |
| #6 | x |  |  |  |
| #7 | x | x |  |  |
| #8 | x |  |  |  |
| #9 | x |  |  |  |
| #10 | x |  |  |  |
| #11 | x |  |  |  |
| #12 |  | x |  |  |

## 6.X	Solution #X: <Solution Title>

### 6.X.0	High-level solution Principles

Editor's note:	Where possible similar/overlapping solution proposals should be documented as a single solution in the TR.

Documentation of more than one solution from the same company for the same KI is allowed but it shouldn't be expected that all of them will be documented in the TR.

### 6.X.1	Description

Editor's note:	This clause will describe the solution principles and architecture assumptions for corresponding key issue(s). Further (sub-)clause(s) may be added to capture details.

### 6.X.2	Procedures

Editor's note:	This clause will describe the high-level procedures and information flows for the solution.

### 6.X.3	Impacts on Services, Entities and Interfaces

Editor's note:	This clause captures impacts on existing services, entities and interfaces.

## 6.1	Solution #1: UP based IMS voice call over NB-IoT NTN via GEO satellite using a default EPS bearer for both IMS signalling and voice data

### 6.1.0	High-level solution Principles

This solution aims to resolve Key Issue #1, " Support of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC " with the following key principles

-	The UE negotiates with MME on its capability of voice call support over NB-IoT.

- 	The default EPS bearer is used for both IMS signalling and voice data transmission.

-  Bearer modification procedure for the QoS update or resource reservation is skipped or pre-initiated before the call setup in case that voice call would be setup over NB-IoT NTN via GEO satellite.

### 6.1.1	Description

In this proposal, it  is assumed to provide IMS voice services via the default EPS bearer. i.e. IMS signalling and IMS voice data are served via the default EPS bearer, which can be updated its non-GBR QoS characteristics if required. Such QoS update is performed before IMS call setup procedure if UE is attached and IMS registered by NB-IoT(GEO).

Editor’s Note: overhead analysis for this approach is to be updated

### 6.1.2	Procedures

#### 6.1.2.1 Attach or TAU procedure negotiating Voice call support over NB-IoT(Geo)(GEO):

On camping NB-IoT(GEO), UE who wants to use voice call service over the NB-IoT(GEO), negotiate its capability of voice call support over NB-IoT.

Moreover, UE to use voice call service over the NB-IoT(GEO) needs to include the IE "Voice domain preference and UE's usage setting" as the voice domain preference = “IMS PS Voice only” and UE’s usage setting = voice-centric.

For IMS service, an IMS dedicated APN is assumed, which is well-known or pre-configured for voice call over NB-IoT(GEO). So the UE should execute UE requested PDN connectivity procedure for the IMS dedicated APN.

However, due to restriction of NB-IoT RAT that dedicated EPS bearer is not used and GBR bearer is not used, we assume the default EPS bearer is used for both IMS signalling and voice data.

#### 6.1.2.2	Default EPS bearer for both IMS signalling and voice data

For IMS service, we assume the default EPS bearer is used for both IMS signalling and voice data.

Editor’s Note:  It is FFS whether or how to differentiate QoS/priority between IMS signalling and voice data.

So in case that UE registers IMS as access type as indicating NB-IoT NTN via GEO satellite, if the voice call is setup b/w caller and callee, the resource reservation in EPS for voice call should be handled specifically;

P-CSCF requests the resource reservation for voice call traffic(i.e. GBR traffic) over NB-IoT(GEO) to PCRF, but P-GW considers as it does not allocate dedicated & GBR bearer for the voice call traffic(i.e. skip the resource reservation) or, if required, performs bearer modification procedure for the default EPS bearer to upgrade its QoS even as a non-GBR bearer.

Editor’s Note: It is FFS whether alternative approach(e.g. P-CSCF provides indication of voice call over NB-IoT(GEO)) is considered.

Moreover, if the expected resource for the voice call over NB-IoT(GEO) is static and need upgrade QoS of the default EPS bearer for the voice call traffic, in order to lessen the call setup time such bearer modification procedure can be initiated before IMS call setup procedure.

### 6.1.3	Impacts on Services, Entities and Interfaces

UE:

- Support capability of voice call support over NB-IoT.

- Set "Voice domain preference and UE's usage setting" when camping on NB-IoT(GEO).

- Use default EPS bearer for both  IMS signalling and IMS voice data over NB-IoT(GEO) access.

MME:

- Support capability of voice call support over NB-IoT.

- Initiate bearer modification procedure for resource reservation/QoS update for the default EPS bearer before IMS call setup.

P-GW:

- allocate IMS signalling and voice mapped to the default EPS bearer.

- ignore the GBR characteristics of IMS voice call data traffic for UE if provided(i.e. skip the bearer modification procedure), or instead perform bearer modification procedure for QoS update for non-GBR characteristics if required

Initiate bearer modification procedure for resource reservation/QoS update for non-GBR characteristics of the default EPS bearer before IMS call setup.

## 6.2	Solution #2: Supporting establishing EPS bearers for IMS voice via user plane with IP type PDN connection

### 6.2.0	High-level solution Principles

This solution aims to resolve KI#1 by supporting the IMS voice call using the user plane of NB-IoT (GEO) satellite access, the following principles are followed in this solution:

-	UE camps on an eNB that supports NB-IoT (GEO) satellite access

-	The UE indicates its support of IMS voice over NB-IoT in attach/TAU request

-	The MME indicates IMS voice over PS session is supported to UE.

-	UE requests to establish an IP type PDN connection with IMS APN via user plane.

-	IMS signalling is transmitted via the default EPS bearer.

-	IMS voice data is transmitted via a dedicated EPS bearer

-	The dedicated EPS bearer may be established together with the default EPS bearer.

### 6.2.1	Description

This solution aims to resolve KI#1 by supporting the IMS voice call over NB-IoT (GEO) via one IMS PDN connection, with default and dedicated bearers as existing IMS voice support over WB-E-UTRAN.

Figure 6.2.2-1: IMS signaling via default EPS bearer and voice data via a dedicated EPS bearer

-	The PDN connection is based on IP type, therefore, the protocol stack could be as follows:

Figure 6.2.2-3: Protocol stack for IMS signalling

Figure 6.2.2-4: Protocol stack for IMS voice data

### 6.2.2	Procedures

#### 6.2.2.1	Attach and IMS PDN connection establishment

The following figure depicts the attach and PDN connection establishment procedure for IMS session over the UP-based NB-IoT (GEO) deployment.

Figure 6.2.2.1-1: Attach and IMS PDN connection establishment Procedure

1.	UE camps on a GEO eNB cell.

2. 	The UE sends Attach Request to the MME, where the eNB indicates RAT type as NB-IoT (GEO) satellite access. During attach, the UE indicates its support of IMS voice over NB-IoT, and optionally an ESM container is included where the PDN type is set as IP.

3.	Same as step 3-6 as specified in clause 5.3.2.1 in TS 23.401 [5].

The UE may send the IMS APN to MME in this step. It is up to the operator’s policy whether the well-known IMS APN is used as default APN in the attach procedure. Using well-known IMS APN during UE attach can reduce the time for UE to be ready for IMS voice call and reduce the needed resources (e.g. number of DRBs), but consequently there is risk that UE may not get data service or even not authorized to attach to the network.

4.	The MME verifies with HSS if the subscription data allows the UE to use IMS voice over NB-IoT (GEO), and indicates IMS voice over PS session is supported to the UE in attach accept.

5.	If an ESM container was not included in the Attach Request, Step 5 is skipped. Otherwise, step 12-16 as specified in clause 5.3.2.1 in TS 23.401 [5] are executed to establish the default EPS bearer for the IMS signalling, the P-CSCF address is provided to the UE when activating the default bearer.

One optimization to reduce the call setup time is to pre-establish the dedicated bearer prior to the IMS voice call setup, i.e. the MME may indicate the APN, Access Info, UE capability and subscription of voice over GEO to the P-GW in order to allow the P-GW to determine to set up dedicated EPS bearer for the IMS voice call, and the P-GW establishes a dedicated EPS bearer for IMS media plane in association with the establishment of the default bearer. Depending on how the packet filter for the dedicated bearer is handled, there are 3 alternatives:

-	Alt#1: the P-GW does not provide TFT containing uplink packet filters to the UE for the dedicated bearer and later during call setup procedure, the UE filters/identifies the uplink voice packets based on the port number negotiated during SIP call setup procedure. In this case no RRC messages are used between UE and eNB.

-	Alt#2: the P-GW creates uplink packet filter (see 3GPP TS 23.060 [12] clause 15.3.3.4 for examples) effectively stopping any packet flow in the uplink direction in this pre-established dedicated bearer for voice media, and during call setup procedure, the P-GW initiates a dedicated bearer modification procedure (see 3GPP TS 23.401 [5] clause 5.4.3) for updating the TFT. In this case two RRC messages (direct transfer) are used between UE and eNB.

-	Alt#3: The MME selects a P-GW with pre-configured TFT information. Both the uplink and downlink packet filters of the TFT for dedicated EPS bearer specific for GEO is determined based on operator policy or implementation (e.g. (list of) AGW IP address, or range of destination IP address and a range of desition port numbers) that will be provided to the UE during the dedicated bearer pre-establishment. In this case no RRC messages are used between UE and eNB during the IMS call setup.

Editors’ note:	The above EPC optimization requires E2E evaluation together with the rest of the system. For example, the IMS SIP singaling transmission currently contributes most of the call setup time (CST), given the SIP signaling message size is larger than RRC message, whether saving of RRC messages would make comparable impact to CST is to be assessed.

Editor’s Note: whether additional information UE capability and subscription of voice over GEO need to be sent to P-GW is FFS.

6-8.	Same as Step 17-22 as specified in clause 5.3.2.1 in TS 23.401 [5] if only default EPS bearer is to be established.

If the P-GW decides to establish a dedicated EPS bearer for the IMS PDN connection in Step 5,  a Dedicated Bearer Setup Request with the corresponding the EPS Bearer QoS, the UE-AMBR etc will also be included in the Initial Context Setup Request, the radio bearer establishment of the default and dedicated bearer for IMS is performed in the same RRC message.

If attach without PDN connection is enabled and used between UE and network (i.e. an ESM container is not included in Attach Request), step 5 is skipped and step 6/7/8 is replaced with Downlink NAS transport/RRC direct transfer, and the following procedures proceed:

9.	The UE sends a PDN Connectivity Request to MME to indicate the requeste the PDN type is IP, and the APN is IMS.

10.	Same as step 5.

11-13.	Same as Step 7-10 as specified in clause 5.10.2 in TS 23.401 [5].

If the PDN GW decides to establish a dedicated EPS bearer for the IMS PDN connection in Step 10,  a Dedicated Bearer Setup Request with the corresponding the EPS Bearer QoS, the UE-AMBR etc will also be included in the S1-AP Bearer Setup Request, the radio bearer establishment of the default and dedicated bearer for IMS is performed in the same RRC message.

The default EPS bearer for IMS signalling is established, and optional the dedicated EPS bearer for IMS voice is also established.

#### 6.2.2.2	MO call

The following figure depicts the mobile originating procedure for IMS session establishment over the UP-based NB-IoT (GEO) deployment.

Figure 6.2.2.2-1: MO call Procedure

1.	The MO UE camps on the eNB that supports NB-IoT (GEO) satellite access and performs attach procedure and establishes the PDN connection for IMS service. The detailed procedure can refer to clause 6.2.3.1. Optionally, the dedicated EPS bearer for IMS voice can be established.

2.	The MO UE performs IMS registration procedure as specified in clause 5.2.2 of TS 23.228 [6], which is via the default EPS bearer of the IMS PDN connection. P-CSCF can obtain RAT type of the IP-CAN by subscribing to the IP-CAN Type Change with PCRF during the SIP registration, as described in 3GPP TS 29.213 [11] Annex B.1a.

3-4.	The MO UE initiates a voice call using SIP INVITE message with an SDP offer towards the P-CSCF, which is over the default EPS bearer of the IMS PDN connection. The P-CSCF forwards the SIP INVITE message towards IMS core and receives SIP 183 response with an SDP answer to the SIP INVITE message as specified in clause 5.6 of TS 23.228 [6].

NOTE:	Any IMS optimizations, including compression or substitution mechanisms, will be addressed in Key Issue#2 study.

5-6.	The P-CSCF sends diameter AAR according to the SDP offer and the SDP answer received to the PCRF as specified in clause 8.1 of TS 29.211 [10]. The PCRF sends a PCC decision provision (QoS policy) message to the PDN GW as specified in step 1 of clause 5.4.1 in TS 23.401 [5].

7.	If a dedicated EPS bearer is pre-established in step 1, the QoS requirement for voice data is configured for the dedicated EPS bearer. The packet filter will be updated based on the alternatives listed in clause 6.2.3.1 Step 5.

If the dedicated EPS bearer is not established in step 1, the PDN GW triggers the dedicated EPS bearer creation procedure as specified in step 2-11 in clause 5.4.1 of TS 23.401 [5], where 3 RRC messages (reconfiguration/complete/direct transfer) are used between UE and eNB.

Editor’s note: whether and how to resolve the conflict with maximum 2-DRB restrion if concurrently supporting data and IMS services for NB-IoT UEs is FFS and needs coordination with RAN WGs.

Editor’s note: which QoS parameters are applicable for the dedicated EPS bearer is FFS and need coordination with RAN WGs.

8-9.	The PDN GW responds to the PCRF as specified in step 12 in clause 5.4.1 of TS 23.401 [5]. The PCRF send diameter AAA to the P-CSCF as specified in clause 8.1 of TS 29.211 [10].

10.	The P-CSCF sends the SIP 183 response with the SDP answer to the MO UE, which is over the default EPS bearer of the PDN connection.

For Alt#1, the UE updates the packet filter locally based on the SDP negotiation result.

11.	The procedure is continued and the IMS session is established.

The IMS voice data are transmitted over the dedicated EPS bearer.

#### 6.2.2.3	MT call

The following figure depicts the mobile terminating procedure for IMS session establishment over the UP-based NB-IoT (GEO) deployment.

Figure 6.2.2.3-1: MT call Procedure

The same procedures as in clause 6.2.2.2, apart from the following steps:

3-4.	The UE-1 receives an incoming voice call using SIP INVITE message with an SDP offer, which is over the default EPS bearer of the PDN connection. The UE sends SIP 183 response with an SDP answer to the SIP INVITE message as specified in clause 5.7 of TS 23.228 [6], which also is over the default EPS bearer of the PDN connection.

10.	The P-CSCF sends the SIP 183 response with the SDP answer to the IMS core.

The IMS voice data are transmitted over the dedicated EPS bearer.

### 6.2.3	Impacts on Services, Entities and Interfaces

PGW:

-	Support determination of dedicated EPS bearer establishment based on IMS APN, and NB-IoT (GEO) satellite access.

-	Support establishing dedicated EPS bearer during Create Session, preconfiguring the packet filter or updating the packet filter locally during call set up, or,

-	Support creating the dedicated EPS bearer for voice for UP-based NB-IoT (GEO).

MME:

-	Support verification of IMS voice call subscription considering NB-IoT (GEO) satellite access, UE preferences.

-	Support indicating IMS Voice over PS session supported to UE.

UE:

-	support creation of the dedicated EPS bearer for voice, and optionally together with during the attach or PDN connection establishment procedure.

-	May support update packet filter locally based on the SDP negotiation result.

-	Indicating support of IMS voice over NB-IoT in Attach/TAU request.

RAN:

-	Support dedicated bearer establishment.

HSS:

-	Subscription data indicating whether IMS voice over NB-IoT (GEO) is allowed.

## 6.3	Solution #3: IP Type IMS PDN connection over User Plane with NIDD voice transfer between UE and PGW.

### 6.3.0	High-level solution Principles

This solution is to address Key Issue #1.

This solution uses a single IMS APN PDN connection with two EPS bearers. The IMS PDN is IP type (e.g., IPv4, IPv6 or IPv4v6) PDN Connection. This solution is a User Plane based solution, i.e., both the default bearer and the dedicated bearer are using S1-U data transfer.

In order to reduce the header overhead to the most extreme level,

-	it is proposed to transmit the IMS voice traffic as non-IP (NIDD) traffic over dedicated EPS bearer; and

-	the PGW will translate the NIDD voice packets to IP/UDP/RTP packets before forwarding them to the IMS eneity in the DN, e.g., the AGW (AGW is not aware of the NIDD transport between PGW and UE); and

at the same time in order to reduce the impact to the IMS control plane ("I"MS network is designed based on IP) and IMS entity in the IMS DN, e.g., P-CSCF,

-	it is proposed to transmit the IMS Signalling (e.g., SIP INVITE…etc) over default EPS bearer using IP.

### 6.3.1	Description

Figure 6.3.1-1: IP Type IMS PDN connection over User Plane with NIDD voice transfer between UE and PGW

NOTE 1	Assuming data rate is 1Kbps and voice codec rate is 0.7Kbps, then there is only 0.3Kbps remaining for voice packet "header" overhead. Assuming ptime = 80ms, which means there will be 12.5 voice packets per second to consume the 0.3Kbps remaining bandwidth, which means each voice packet header part can only use 300bits/12.5 == 24 bits, i.e., each voice packet can only tolerate maximum 3 Bytes header size, these overhead are shared by multiple layers including RTP/UDP/IP/NAS(if go with CP)/RRC(if go with CP)/PDCP(if go with UP or go with CP after DRB is established)/RLC/MAC headers.

The IMS PDN (for voice over GEO) is used to transmit both IMS signaling and IMS voice.

-	For IMS signalling:

-	the main consideration is to minimize the impact to the IMS control plane network ("I"MS network is designed based on "IP") so the default EPS bearer for IMS PDN connection is configured as IP transmission (i.e., IMS Signalling over IP, no special handling).

-	For voice packets, on the other hand, it is proposed to use non-IP (NIDD) type transmission.

Editor’s Note:	In the context of using "Non-IP" voice packet transfer, only one call or more than one call needs to be supported is FFS based on SA1 discussion.

Editor’s Note:	In the context of using "Non-IP" voice packet transfer, what are the essential RTP header fields for the minimum information that need to be provided in RTP for IMS voice over GEO is FFS based on SA4 discussion.

Thus the ideal IMS PDN configuration for IMS voice over GEO would be:

-	the PDN Type for this IMS PDN is IP Type; and

-	a dedicated EPS bearer is configured to use NIDD to transmit voice packets.

### 6.3.2	Procedures

#### 6.3.2.1	Establishment and Usage of IP Type IMS PDN connection over User Plane with NIDD voice transfer between UE and PGW.

In order to support:

-	IP Type IMS PDN connection over User Plane with NIDD voice transfer between UE and PGW, which support to use

-	IP (IP(v4), IPv6 or IPv4v6) type traffic for default EPS bearer; and

-	non-IP type traffic for dedicated EPS bearer;

the UE supports the this feature indicates support of "IP and non-IP Hybrid Transfer" in UE network capability in Attach request, and the MME supports the "IP and non-IP Hybrid Transfer" indicates to accept to use it in Attach accept; and during Dedicated EPS bearer context activation procedure the network indicates, for a dedicated bearer to be activated for voice, the traffic type to be sent over the bearer is non-IP traffic.

#### 6.3.2.2	NIDD voice transfer between UE and PGW

##### 6.3.2.2.1	Uplink Voice Data Transmission using NIDD over dedicated EPS bearer

###### 6.3.2.2.1.1	UE side consideration

For the uplink voice packets to be sent over a dedicated bearer using NIDD, i.e., without IP/UDP header the existing Packet filter component type identifier (as specified in TS 24.008 [13] clause 10.5.6.12) is not able to indicate the "voice traffic" needs to be steered to the dedicated EPS bearer context, thus a new Packet filter component type identifier may be introduced, e.g., "(IMS) voice traffic type", to indicate the UE to steer NIDD voice traffic to the dedicated EPS bearer context. The UE sends the "Voice with RTP info (e.g., 1 Byte SN) packet" over the dedicated EPS bearer context associated with this "(IMS) voice traffic type” Packet filter component type identifier.

###### 6.3.2.2.1.2	Network side consideration

When the PGW receives uplink voice packets from the dedicated EPS bearer, the PGW converts the "Voice with RTP info (e.g., 1 Byte SN) packet" to "Voice over RTP/UDP/IP packet".

In order to build the context of the conversion operation, when the PDN GW triggers the dedicated EPS bearer creation procedure as specified in step 2-11 in clause 5.4.1 of TS 23.401 [5], the PGW also builds and stores the context (e.g., IP(es) and Port(s)), based on information received from PCRF, in order to perform the conversion, i.e., to construct the RTP/UDP/IP headers for the received uplink voice packet and then forward the conversed voice packets (with RTP/UDP/IP header) to the DN.

Editor's note:	Details regarding how the PGW performs the conversion is FFS.

Editor's note:	Whether a new functionality in the PGW is needed for the conversion between NIDD voice and IP/UDP/RTP voice is FFS.

##### 6.3.2.2.1	Downlink Voice Data Transmission over NIDD over dedicated EPS bearer

For the downlink voice packet, the PGW may still leverage the current DL TFT as usual, but for DL traffic steered to the NIDD dedicated EPS bearer the PGW converts the DL Voice over RTP/UDP/IP packets to Voice with RTP info (e.g., 1 Byte SN) packet. The conversion can be simply removing the RTP/UDP/IP headers and insert 1 Byte SN value (e.g., use the 1 Byte SN value originally indicated in the removed RTP header) as the converted header part of the voice packet before forwarding the voice packet to SGW.

### 6.3.3	Impacts on Services, Entities and Interfaces

Editor's note:	Whether a new GBR or non-GBR QCI is needed for the dedicated EPS bearer used for voice traffic is FFS. Whether an existing GBR or non-GBR QCI can be used for the dedicated EPS bearer used for voice traffic is FFS.

UE:

-	support dedicated EPS bearer for NB-IOT.

-	indicate support of "IP and non-IP Hybrid Transfer" to the MME.

-	new Packet filter component type identifier, e.g., “(IMS) voice traffic type”, to let the UE indicated to steer NIDD voice traffic to the dedicated EPS bearer context.

-	transfer IMS voice packet over dedicated bearer without adding IP/UDP/RTP header.

-	transfer IMS voice packet over dedicated bearer with minimum RTP information in the header (e.g., 1 Byte SN).

Editor’s Note:	In the context of using "Non-IP" voice packet transfer, only one call or more than one call needs to be supported is FFS based on SA1 discussion.

Editor’s Note:	In the context of using "Non-IP" voice packet transfer, what are the essential RTP header fields for the minimum information that need to be provided in RTP for IMS voice over GEO is FFS based on SA4 discussion.

MME:

-	indicates support of "IP and non-IP Hybrid Transfer" to the UE.

RAN:

-	support dedicated EPS bearer for NB-IOT.

-	support NIDD bearer for the "IP and non-IP Hybrid Transfer" PDN connection.

Editor's note:	Detail impacts on RAN, e.g., whether and how to enable/disable RoHC in the RAN for the voice bearer, is FFS.

PGW:

-	support dedicated EPS bearer for NB-IOT.

-	new Packet filter component type identifier, e.g., "(IMS) voice traffic type”, to indicate the UE to steer NIDD voice traffic to the dedicated EPS bearer context.

-	for uplink voice packets, convert the NIDD voice packets into IP packets before forwarding the packets the the IMS DN.

-	for downlink voice packets, convert the IP/UDP/RTP voice packets into NIDD voice packets before forwarding the packets to the UE.

-	building conversion parameters (e.g., source IP, destination IP, source port, destination port) for voice packets by leveraging existing UL and DL TFT parameters used during dedicated EPS bearer creation procedure.

Editor's note:	Details regarding how the PGW performs the conversion is FFS.

Editor's note:	Whether a new functionality in the PGW is needed for the conversion between NIDD voice and IP/UDP/RTP voice is FFS.

IMS network:

-	None

## 6.4	Solution #4: Support of IMS call over NB-IoT NTN via NIDD and I1 protocol

### 6.4.0	High-level solution Principles

This solution addresses KI#1, KI#2, KI#3, and KI#4.

To enable IMS-based voice services over NB-IoT NTN access through GEO satellite connected to the EPC, the following high-level architectural principles are applied.

- The mechanism for separate default DRBs for Signalling and Media

- User Plane NIDD

- SCC-AS architecture in TS 23.292 [15] and I1 protocol for IMS session setup

NOTE: Although responding to the needs of GEO satellite deployments, the architectural principles described herein could also be applicable to NB-IoT NTN in NGSO deployments.

### 6.4.1	Description

#### 6.4.1.1 	General

This solution addresses:

- KI#1 as described in clause 6.4.1.2 and clause 6.4.1.4

- KI#2 as described in clause 6.4.1.3

- The solution also addresses KI#3 and KI#4 by proposing reusing E-CSCF in clause 6.4.2.3 where it is further proposed that existing location services can be used to obtain the UE location (e.g., geographical location) meeting regulatory requirements, when the UE establishes an IMS emergency call over NB-IoT NTN via GEO satellite connecting to EPC.

#### 6.4.1.2	DRBs for signalling and media

It is proposed to establish separate Data Radio Bearers (DRBs) for signaling and voice media packets due to different transport requirements for these two types of packets. Voice packets require consistent jitter and delay handling and may benefit from prioritization or special treatment in satellite links. This separation also aligns with how IMS networks typically handle control and media, enabling better traffic management and potential for differentiated satellite resource allocation.

NB-IoT channels operating over GEO satellites face significant bandwidth constraints, limited to 180 kHz usable bandwidth. Moreover, these constraints restrict the number of simultaneous voice calls over a non-anchor carrier. To maximize transmission efficiency over these narrowband links, it is proposed to use User Plane Non-IP Data Delivery (UP NIDD) for both call control (e.g. call setup, Supplementary Service Control) signaling and voice media.

Table 6.4.1.2-1: Data Delivery Overhead Comparisons

| Bytes | RTP | UDP | IP | PDCP | RRC | NAS | RLC | MAC | BSR | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IPv4 | 12 | 8 | 20 | 1 |  |  | 2 | 1 | 2 | 46 |
| IP with RoHC |  |  | 3 | 1 |  |  | 2 | 1 | 2 | 9 |
| UP NIDD |  |  |  | 1 |  |  | 1 | 1 | 2 | 5 |
| CP NIDD |  |  |  |  | 2 | 11 | 2 | 1 | 2 | 18 |

As shown in Table 6.4.1.2-1 above, UP NIDD provides the lowest protocol overhead, making it especially suitable for constrained and high-latency environments such as GEO-based NTN. UP NIDD, operating over the user plane, also enables the use of AS-level security mechanisms. Although NB-IoT does not support dedicated data radio bearers, the default bearer used by UP NIDD allows for radio-level optimization (e.g., RLC AM for signaling, RLC UM for media), which can be configured for improved reliability and latency control.

IPv4 is not ideal in this context due to the relatively high overhead of protocol headers. IP with RoHC adds additional complexity and CPU processing for both the UE and the network. This makes it less efficient for NB-IoT, where devices are constrained by limited processing capability, power consumption, and memory footprint.

#### 6.4.1.3	Use of SCC-AS architecture for IMS Session Setup

It is proposed to use IMS Service Centralization and Continuity architecture (see clause 4.20 of TS 23.228 [6] and TS 23.292 [15]) for IMS session establishment over NB-IoT NTN GEO links. I1 protocol (as defined in 3GPP TS 24.294 [14]) is designed specifically for this scenario where the bearer path is CS or the transport used for IMS call control is bandwidth constrained, but IMS service control is still required.

The I1 protocol is highly lightweight, with fixed header-length of 7 bytes, making it well-suited for low-bandwidth, high-latency satellite links by significantly reducing transmission time. While originally designed for CS-to-IMS interworking and IMS Centralized Services (ICS), it can be readily adapted for NIDD-to-IMS interworking, as discussed in Clause 6.4.1.3.

NOTE:	I1 protocol is a transport independent protocol, see 3GPP 24.294 [14] clause 1. That is, the I1 protocol I1 session control entities can exchange the I1 protocol messages over any transport-layer connection that connects the ICS UE and the SCC AS. In this solution, we propose to use UDP as transport layer to carry I1 messages.

I1 is already standardized and supported by the Service Centralization and Continuity Application Server (SCC-AS), and since it eliminates the need for a SIP stack on the UE, it also greatly simplifies device implementation.

Support of IMS Service Centralization is specified in TS 23.228 [6] and TS 23.292 [15]. IMS service centralization provides communication services such that all services, and service control, are based on IMS mechanisms and enablers. This solution proposes to leverage IMS centralised services architecture to support IMS services when using NB-IoT NTN access with UP NIDD, as shown in Figure 6.4.1.3-1.

Figure 6.4.1.3-1: Overall Architecture for IMS Voice Call over NB-IoT NTN

The I1 protocol enables sessions that originate from or terminate in the NIDD domain to be anchored and controlled by IMS. This ensures service continuity and consistent behavior across both NIDD and IMS domains.

When a voice call is initiated or received by a user with an IMS subscription and NB-IoT NTN (GEO) access, the session falls back to a User Plane (UP) NIDD-based configuration. In this setup:

- Up to two default bearers are supported on NB-IoT devices per TS 23.401 [5]. Two default DRBs are established per IMS session, one for I1 signaling and one for voice media. This separation allows media traffic to receive differentiated treatment, even in the absence of conventional QoS mechanisms.

- An Interworking Function (IWF), specifically the SCC-AS as defined in TS 23.292 [15], receives session signaling over the I1 protocol and translates between I1 and SIP messages, enabling seamless integration with standard IMS components.

- The media parameters included in the I1 message allow the Media Gateway within the IWF to determine whether transcoding is required. By resolving codec mismatches internally, the system avoids traditional end-to-end codec negotiation, significantly reducing call setup latency, especially important in high-delay satellite environments. The media Gateway also adds RTP header for uplink voice payload or stripe RTP header for downlink voice payload.

- The IWF may include an MSC Server/VLR-like functionalities to handle registration and location update for subscribers that are currently in the area served by NB-IoT NTN access.

NOTE:   CS is not required between UE and MSC. The MSC Server supports functionality to trigger I2 registration procedures and maintain SGs association towards MME for EPS/IMSI attached UE.

Editor's note:	The exact functionality to be supported by IWF is FFS.

The NIDD UP session is anchored at the Service Centralization and Continuity Application Server (SCC-AS), which maintains IMS session control and service logic regardless of the underlying access technology.

All external interfaces of the interworking function comply with existing 3GPP specifications, so no additional standardization is required. Since it operates entirely within the operator’s network domain, it can be deployed without impacting standardized interfaces. In practice, the interworking function can be integrated into the SBC, with the I1 and SIP message translation implemented as a B2BUA (as specified in TS 24.229 [16]), effectively providing a SIP trunk between the interworking function and the IMS core.

For transport, the User Plane uses RLC Acknowledged Mode (AM) for the signaling bearer to ensure reliable delivery, and RLC Unacknowledged Mode (UM) for the media bearer to reduce latency. Both bearers can be configured by the eNB as needed.

Because codec negotiation is handled entirely by the interworking function, there is no need for direct capability exchange between endpoints. This simplifies the UE design and enables a seamless, efficient voice experience across diverse network and device types.

#### 6.4.1.4 	QoS support

NB-IoT does not support dedicated bearer setup with Guaranteed Bit Rate (GBR), which is commonly used in eMBB and LTE networks to support IMS call setup. To enable differentiated treatment of IMS session traffic over NB-IoT NTN via GEO satellite, alternative strategies must be considered.

One approach is to assign an existing or newly defined QCI to voice traffic over these links. This QCI is applied to the default bearer associated with a dedicated APN configured specifically for IMS services over NB-IoT NTN via GEO. The QCI parameters should be optimized to meet the performance requirements of conversational voice in high-latency, narrowband environments.

It is also recommended to configure separate APNs for signaling and media traffic, each using its own default EPS NIDD UP bearer. This enables the core network to prioritize traffic by assigning higher scheduling priority, applying distinct DSCP markings or packet filters, and optionally routing media traffic through a dedicated PGW optimized for low latency or accelerated processing.

At the RAN level, the DRBs established for IMS traffic over NB-IoT GEO should be treated with elevated priority, including preferential scheduling and minimized packet discard during periods of congestion.

### 6.4.2	Procedures

#### 6.4.2.1 	MO Call Procedures

Fig. 6.4.2.1-1 provides an example flow for MO call procedures.

Figure 6.4.2.1-1: Example flow for MO call procedures

- UE-A access IMS service over NB-IoT NTN via GEO with an indication to use IMS over UP NIDD APN

- In step 1a, the UE-A initiates the attach procedure by the transmission of an Attach Request, with PDN connection type “Non-IP”. The Attach Type indicates that the UE requests a combined EPS/IMSI attach. And the network is informed that the UE is capable and configured to use UP NIDD for IMS voice via GEO.

The PDN connection associates a PDN represented by an APN and a UE represented by UE Identity for Non-IP PDN Types as specified in TS 23.401 [5].

In step 1b, steps 3 to 16 of the EPS Attached procedure are performed as specified in TS 23.401 [5].

- In step 2a, the MME sends a Location Update Request towards MSC Server/VLR via SGs.

In step 2b, the MSC/VLR Server enhanced for ICS performs location update, authentication and obtains subscriber data. The VLR updates the HLR with the current location of the UE.

In step 2c, The HLR acknowledges the update, confirming that the UE is now associated with the MSC Server/VLR serving the NB-IoT NTN. A Location Area Update Accept is returned to the MME.

The UE then performs the remaining steps to set up the IMS call via NB-IoT NIDD UP NTN.

- Steps 17 to 26 of the EPS Attached procedure are performed as specified in TS 23.401 [5]. And the EPS Attach procedure is completed with default bearer, with the DRB for IMS I1 signalling. "Delivery using a Point-to-Point (PtP) SGi tunnel”, as defined in 23.401 [5] clause 4.3.17.8.3.3, can be used to deliver the Non-IP data at the SGi interface. Specifically, the tunnel parameters (i.e. destination IP addresses and UDP ports to the SCC AS or Media GW) for SGi PtP tunnelling based on UDP/IP are pre-configured on the P-GW. For uplink Non-IP data, the P-GW forwards the received data to the AS over the SGi PtP tunnel. For downlink Non-IP data, the AS sends the data using UDP/IP encapsulation with the IP address of the UE and the 3GPP defined UDP port for “Non-IP” data.

NOTE: The PtP tunnel and transport protocol for I1 messages are pre-configured and under operator control.

Editor's note:	It is FFS how the PGW route the UL media data when Non-IP PDN Connection Type is used but the destination using IP.

- The MSC Server enhanced for ICS decides to initiate IMS registration for this subscriber if the subscriber is not already registered following steps 4-11 of Clause 7.2.1.2 of TS 23.292 [15].

- With the information of the completion of the first PDN connectivity with NB-IoT NTN UP NIDD, UE initiates the second default bearer for voice payload in this step. The voice payload as encapsulated in the NIDD may include simplified RTP header or without RTP header, with standard RTP header being added back after media gateway in IWF as illustrated in Figure 6.4.1.3-1. QoS mechanisms is outlined in Clause 6.4.1.4.

- In Step 6a, UE-A initiates a call by sending an ICS Call Initiation Request over I1 reference point containing the UE-B address. In step 6b, the SCC AS sends the SCC AS PSI DN for the purpose of UP NIDD bearer setup to the UE over the I1 reference point.

Editor’s note: It is FFS how security association between UE and SCC AS is established without IP.

- UE-A sends an I1 INVITE to the SCC AS with SCC AS PSI (for the purpose of UP NIDD bearer setup) and SDP-MGW, e.g. codec supported, etc.

- The SCC AS invokes a B2BUA, terminating the UE-A Leg and originating a leg for presentation of an IMS session towards UE-B on behalf of UE-A. The SCC AS correlates the signalling session bearer (over UP-NIDD) with the IMS service control signalling session using the SCC AS PSI and creates a SIP INVITE containing the SDP received in the UP-NIDD I1 signalling path. The INVITE request is routed from the SCC AS to the IMS core where standard IMS originated session processing and routes the request onwards to the UE-B.

- Completion of the Service Control Signalling Procedures.

- As described in Clause 6.4.1.3, with the knowledge of both UE-A and UE-B’s codec capabilities, the MGW determines, without further negotiation, whether a transcoding is necessary.

- Media Session continues between two parties UE-A and UE-B.

#### 6.4.2.2 	MT Call Procedures

Figure 6.4.2.2-1: MT Call Procedures

Steps 0-4 in Figure 6.4.2.2-1 are identical to steps 0-4 in Figure 6.4.2.1-1.

- An incoming SIP INVITE is received at the IMS core of the UE-A party from UE-B.

- The IMS core (e.g. S-CSCF) executes terminating initial Filter Criteria and forwards the SIP INVITE to the SCC AS.

- The SCC AS sends a Session Progress response to the IMS, and forwards the Session Progress response to UE-B.

- The SCC AS performs Terminating Access Domain Selection (T-ADS) and if informed of the UE-A capability for I1 during IMS registration, chooses the NIDD UP domain for the setup of the media.

Editor’s note: How exactly T-ADS works at this step if FFS.

- The SCC AS terminates the session from UE-B and establishes a new session over I1 by sending an Incoming Call Request to the UE-A.

The Incoming Call Request may include an indication instructing UE-A to initiate the NIDD UP bearer establishment procedure for a second default bearer for voice payload.

Additionally, the Incoming Call Request may contain the SCC AS PSI DN, allowing the SCC AS to later correlate outgoing service control signalling with the incoming I1 control signalling over the NIDD-UP bearer.

- Triggered by I1 ICS Incoming Call message, the UE initiated a default bearer setup for voice payload. This bearer utilizes the QoS mechanisms outlined in Clause 6.4.1.4. In step 10a, the MGW determines codec selection for media session and perform transcoding if required.

- In steps 11a-11b, UE-A sends a I1 ALERT to the SCC AS, and the SCC AS forwards SIP 180 Ringing message to UE-B. In steps 11c-11d, UE-A sends I1 SUCCESS message to the SCC AS, and the SCC AS forwards a SIP 200 OK message to UE-B in response to the INVITE received at step 6.

- Media Session continues between two parties UE-A and UE-B.

#### 6.4.2.3 	Emergency Call Procedures

Editor's note:	It is FFS how to perform emergency registration using the MSC-based I6 interacting with E-CSCF, as well as SCC-AS.

If the SCC-AS detects an IMS emergency call over NIDD-UP via GEO, e.g. through emergency service indication in the initial I1 message or based on the dialed emergency number from an NB-IoT UE accessing via GEO NTN, it may interwork with IMS for routing the emergency session to the PSAP.

If emergency registration is performed using the I1 interface toward the SCC-AS, and the SCC AS then proceeds to send the emergency session initiation request to the E-CSCF in the IMS core using standard SIP signaling, by following the architecture in Figure 6.4.1.3-1.

If the SCC AS is required to handle anonymous emergency calls (e.g., due to local regulations), it shall skip the IMS registration step and directly send the anonymous emergency session initiation to the E-CSCF using SIP. The identity used may be derived from configuration or replaced with a temporary anonymous identifier as per IMS emergency call procedures.

If the emergency session is tagged with eCall indication from the UE (e.g., via a special I1 information element), the SCC-AS may include the IMS eCall indication in the SIP INVITE to the E-CSCF. The Minimum Set of emergency related Data (MSD) shall be transferred via the User Plane NIDD media bearer. The SCC AS and IMS shall transport the MSD data transparently end-to-end from UE to the PSAP, maintaining the format generated by the UE.

For a UE that has established a NIDD bearer with the SCC AS, but has not yet performed any form of logical registration for IMS service control (e.g., via I1), the SCC AS may follow a behavior equivalent to a non-enhanced interworking node, potentially routing the call using pre-configured emergency handling mechanisms or default service logic.

It is further proposed that existing location services can be used to obtain the UE location (e.g., geographical location) meeting regulatory requirements, when the UE establishes an IMS emergency call over NB-IoT NTN via GEO satellite connecting to EPC.

### 6.4.3	Impacts on Services, Entities and Interfaces

Editor's note:	Further impacts are FFS.

MSC Server/VLR:

Deployed in an Interworking Function to maintain SGs association towards MME for EPS/IMSI attached UE.

MME:

Maintain SGs association towards MME for EPS/IMSI attached UE.

UE:

Support of I1-like client, but it could be done with APP via software implementation. It does not require the existence of SIP stack.

SCC AS:

Support of creating SIP containing the SDP received in the NB-IoT NTN bearer path.

## 6.5	Solution #5: Support of IMS voice NB-IoT NTN connected to EPC using control plane bearers and new SRBs

### 6.5.0	High-level solution Principles

This solution addresses Key Issue 1 and supports IMS voice call GEO via NB-IoT NTN connected to EPC using CP bearers.

This solution is underpinned by the usage of a new SRB (Signaling Radio Bearer) dedicated to transmit voice packets using Control Plane CIoT EPS Optimisation which needs to be defined by RAN WGs.

### 6.5.1	Description

#### 6.5.1.1	Requirements

To support IMS voice service, two service data flows (SDFs) need to be supported: one for IMS signalling and the other for IMS voice packets. The voice packet flow is a real-time flow with stringent delay requirements. Due to the high round trip time over a GEO satellite link, this flow does not lend itself to acknowledged mode transmission on the radio. The IMS signalling flow on the other hand requires high reliability and can tolerate some delay.

Considering the above, when Control Plane CIoT EPS optimisation is used as defined in TS 23.401 [5], these two SDFs need to be carried over two separate signaling radio bearers (SRBs) over the air in order to meet the different QoS requirements.

The transport of the voice packet SDF should be performed with the minimum protocol overhead possible in order to minimize the delay.

#### 6.5.1.2	Network architecture

Figure 6.5.1.2-1 below shows high level architecture for NB-IoT NTN connected to EPC using EPS bearers that use Control Plane CIoT EPS optimisation. The details of the mapping of the SDFs to EPS bearers are described in more detail in clause 6.5.1.4.

Figure 6.5.1.2-1: Architecture for NB-IoT NTN connected to EPC using CP CIoT

#### 6.5.1.3	PDN connection types

The PDN type of the PDN connection can be IP or non-IP. For the set of combinations, the following options are possible based on today’s standards:

a)	Both SDFs are transported over the same PDN connection with PDN type IP. In this case:

-	The IMS signalling flow uses SIP/UDP/IP stack;

-	The voice packet flow uses RTP/UDP/IP stack;

b)	Both SDFs are transported over the same PDN connection with PDN type non-IP.

-	The voice packet flow uses RTP/NIDD stack;

Editor's note:	The protocol stack for the IMS signalling flow for this case is FFS.

c)	IMS signalling flow and the voice packet flows use different PDN connections. Namely, the IMS signalling flow uses PDN type IP and the voice packet flow uses PDN type non-IP (RTP/NIDD stack).

#### 6.5.1.4	Bearer mapping

##### 6.5.1.4.1	Bearer mapping (General)

The IMS sigalling SDF and the voice packet flow have different transport requirements in terms of delay and reliability. For example, the voice data flow cannot use RLC Acknowledged Mode (AM), while the IMS signalling flow uses RLC AM. Therefore, the voice packet flow requires a separate SRB from the SRB used for IMS and NAS signalling. The architecture with the mapping of SDFs onto EPS bearers and then signalling radio bearers is shown in Figure 6.5.1.4-1. This architecture includes a new SRB dedicated to transporting the voice packet flow (SRBx).

Figure 6.5.1.4-1: Bearer mapping for IMS voice over GEO via NB-IoT NTN connected to EPC using CP CIoT

Editor’s note:	Whether a separate SRB is needed for the IMS signalling flow depends on RAN and is FFS.

Editor's note:	The handling of the PDN connection(s) for the IMS signalling and the voice packet flow is FFS.

##### 6.5.1.4.2	Bearer mapping when using IP type PDN connection

In IMS voice sessions for E-UTRAN/EPS the establishment of the dedicated EPS bearer that carries the voice packets is established “on demand” using dynamic PCC and the QCI-1 that is used for that purpose is GBR. Considering that NB-IoT does not support GBR bearers and establishment of dedicated EPS bearer in tandem with the IMS call setup will introduce additional delays some different arrangement is needed. What is proposed is:

- The default EPS bearer of the IMS PDN connection is that of QCI-5 and is used to carry IMS signalling;

- An additional dedicated EPS bearer is established in combination with or directly after the default EPS bearer activation at Attach or UE requested PDN connectivity procedures to carry the voice packets. This EPS bearer is also non-GBR and possibly needs a new QCI to be defined;

Editor’s note:	Whether a new non-GBR QCI is needed for the EPS bearer used for voice traffic is FFS.

- If combined bearer activation is used, following the steps of TS 23.401 [5], Annex F, in steps 7a/7b depending on whether the Attach or PDN connection establishment procedure is used MME sends the S1-AP Initial Context Setup Request or S1-AP Bearer Setup Request message to the eNodeB, including the NAS parts for both the Attach Accept message of the Attach procedure and the Bearer Setup Request of the Dedicated Bearer Activation Procedure. Otherwise, a separate dedicated EPS bearer is established after the Attach or PDN Connection establishment.  Similarly, in message 8, RRC Reconfiguration is used. Neither of these S1-AP and RRC messages and procedures are currently supported for NB-IoT UE, MME and eNodeB supporting only Control Plane CIoT EPS optimisation.

Editor's note: Security Issues regarding the support of RRC Reconfiguration for Control Plane CIoT EPS optimisation need to be checked with SA3.

They are though supported for UE, MME and eNodeB supporting S1-U and can be re-used as follows:

-	In accordance with the steps outlined in TS 23.401 [5], the “Initial Context Setup Request” or Bearer Setup Request from the MME to the eNodeB shall be able to indicate a request to establish dedicated SRB(s) and the associated EPS bearer id(s).

-	The subsequent “RRC Connection Reconfiguration Request” from the eNodeB towards the UE shall include the associated EPS bearer identities for the corresponding SRB(s).

Editor’s Note: if the new SRB is required in normative phase, the procedure to add/modify/delete a new SRB is up to RAN2 decision

- For the dedicated EPS bearer for the voice traffic the detailed UL and DL TFT containing all the 5-tuple information cannot be decided until the IMS session is established. At the beginning i.e. at time of Attach/PDN connection establishment, a “block all” filter shall be used for both the UL and DL. At the time of the IMS call setup, the DL TFT can be updated based on TS 23.203 [17], clause 7.4.2. At the time of the call setup the PDN GW initiated bearer modification without bearer modification procedure can be executed towards the UE using Downlink NAS Transport. The UL TFT in order to reduce the “over the air” signalling, the UE can locally derive based on the SDP information exchanged at the time of IMS call setup and replace the previously use “block all” packet filter.

Editor’s note:	 What information needs to be included in the Downlink NAS Transport and how to optimize the overhead is FFS.

##### 6.5.1.4.3	Bearer mapping when using non-IP type PDN connection

An additional challenge in supporting voice traffic over NB-IoT NTN via GEO satellite lies in fulfilling the data rate and call setup time requirements, taking into account the increased round-trip delay inherent to GEO satellite links. To address this, one alternative approach would be the use of a non-IP PDN type, rather than an IP-based PDN. This would for instance reduce the protocol overhead (see clause 6.5.1.5). The use of a non-IP PDN type could apply regardless of whether Control Plane CIoT EPS Optimization is used or not.

However, in case of non-IP PDN type, the TFT filters can no longer use IP 5-tuple (i.e. source/destination IP addresses port numbers and transport protocol) to map traffic to separate EPS bearers. Hence, the following new functionalities are proposed:

-	The separation of IMS signalling and voice media shall be based on the traffic type (SIP or I1 vs. RTP/RTCP) in the UL direction. The UE shall map the UL traffic based on the traffic type to the corresponding EPS bearer identities and shall use the corresponding radio bearer(e.g. SRB as per this proposal) for the EPS bearer as configured in the RRC.

-	In DL direction, the PGW shall map the DL traffic using DL TFT as a PDN type PGW as per existing mechanism for IP PDN type i.e. using IP 5-tuple (i.e. source/destination IP addresses port numbers and transport protocol).

Editor's note:        Details regarding how the PGW performs the IP and NIDD conversion is FFS.

#### 6.5.1.5	Protocol overhead for the voice packet flow

The protocol overhead for the voice flow (SRBx) is discussed below. This overhead is per voice packet.

MAC layer overhead: MAC layer overhead is one byte, since there will be only one MAC PDU per TBS.

Editor's note:   Whether to use "Single MAC PDU per TBS" is FFS based on SA4 and RAN2 discussion.

RLC layer overhead: For RLC Unacknowledged Mode (UM) it is one byte. For RLC Transparent Mode (TM) it is zero.

Editor's note:	Whether RLC UM or RLC TM is used is FFS based on RAN2 feedback.

RRC overhead: The only overhead due to RRC encapsulation over SRBx is the length indicator for the NAS PDU, which is up to one byte. The length indicator could potentially be also eliminated (see section 6.5.1.6).

Editor's note:	Whether RRC encapsulation overhead can be eliminated is FFS.

NAS layer overhead: In Rel-19, CT1 specified a new NAS message for data transfer over NAS a reduced NAS layer overhead of 3 bytes. The NAS layer overhead could potentially be reduced to zero (see section 6.5.1.6 ).

Editor's note:	Whether further NAS layer overhead reduction is possible is FFS.

NAS security overhead: NAS layer security overhead consists of 4 bytes for MAC (integrity protection) and one byte for SN. However, VoLTE is currently transported without integrity protection in all commercial deployments since User Plane Integrity Protection in E-UTRAN/EPC even though it is standardized is not commercially deployed. If accordingly user plane integrity protection is not required, the NAS security overhead could be one byte.

Editor's note:	Whether voice packets can be transported via SRBx without integrity protection is FFS and needs to be discussed with SA3.

Editor’s Note: It is FFS whether the assumption regarding the best case of NAS protocol overhead is possible. It needs coordination with SA3 and CT1

IP/UDP/RTP overhead:

-	For IP PDN type: RoHC overhead after RoHC context establishment (full compressed packet) is 3 bytes. However, the IP/UDP/RTP overhead without RoHC (e.g. during RoHC context (re-)establishment) is at least 30 bytes.

-	For non-IP PDN type (RTP/NIDD), the RTP overhead is one byte (for RTP sequence number).

Editor's note:   Whether 1 octet of RTP header (RTP sequence number) is sufficient needs to be checked with SA4.

Table 6.5.1.5-1 summarizes the overhead size for different protocol layers and provides the total overhead for the best case and the worst-case scenario.

Table 6.5.1.5-1 Protocol overhead for voice packet flow over the control plane

| Protocol layer | Description | Best Case | Worst case | Comments/impacts |
| --- | --- | --- | --- | --- |
| MAC | Single MAC PDU per TBS | 1B | 1B | Supported in the specs. |
| RLC | TM or UM | 0 (TM) | 1B (UM) | 0 bytes requires spec support for RLC TM/UM for SRB. |
| RRC | RRC encapsulation of the NAS PDU | 0 | 1B | For zero RRC overhead see section 6.5.1.6. |
| NAS protocol | NAS message overhead | 0B | 3B | CT1 work in Rel-19 reduced NAS OH to 3 bytes. For zero NAS overhead see section 6.5.1.6. |
| NAS security | MAC and NAS SN | 1B | 5B | 1B requires MAC suppression. |
| IP/UDP/RTP |  | 1B | 3B-30B | For non-IP PDN type: 1B for RTP SN. For IP PDN type: 3B for RoHC. 30B during RoHC context (re-)establishment. |
| TOTAL |  | 3B | 14B-41B | Key factors affecting OH: - MAC (integrity protection) suppression - IP vs non-IP PDN type - For IP PDN type: RoHC OH during RoHC context (re-)establishment. - SRB identification over S1-AP (see section 6.5.1.6) - NAS PDU size pre-negotiation over SRBx (see section 6.5.1.6) |

The overhead data rate depends on the packet rate. For example, for a 3B and a 6B overhead per packet and 50 packets per second (one packet each 20 ms), the overhead data rate would be 1200 bps and 2400 bps, respectively.

#### 6.5.1.6	Protocol overhead reduction for the voice packet flow

It is possible to leverage the SRBx dedicated to the transport of the voice SDF to eliminate both the RRC and the NAS layer overhead. The NAS layer overhead discussed in this subclause does not include the NAS security overhead.

##### 6.5.1.6.1	RRC overhead reduction

Since SRBx is dedicated to the transport of the voice SDF, no RRC messages are needed over SRBx, i.e. SRBx can carry the DedicatedInfoNAS IE (NAS PDU) directly.  This way, the RRC overhead could be reduced to zero. In other words, the voice packet would be identified over the air using the logical channel ID of the SRBx at the MAC layer.

Editor's Note:	Whether voice packets could be identified over the air using the logical channel ID of the SRBx at the MAC layer is FFS. This needs to be discussed with RAN2.

Editor’s Note: It is FFS whether the assumption that the voice and silent packets have constant size for a given codec rate stands. This may need coordination with SA4.

##### 6.5.1.6.2	NAS layer overhead reduction

Since SRBx is dedicated to the transport of voice pace SDF, any NAS PDU received by the MME over SRBx contains a voice packet. Therefore, NAS layer overhead is not needed to inform the MME about the content of the NAS PDU received over SRBx. To associate the NAS PDU with SRBx, the eNB could insert an indication e.g. signalling bearer identity in the S1-AP NAS transport messages (similar to how the legacy EDT session indication IE indicates the association with an EDT session in the S1-AP Initial UE message).

### 6.5.2	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

### 6.5.3	Impacts on Services, Entities and Interfaces

Editor's note:	This clause captures impacts on existing 3GPP nodes and functional elements.

MME:

-	Based on the UE support for dedicated SRB(s), and possibly based on the user subscription, decides to utilize dedicated SRB(s) with the UE.

-	Supports establishing of dedicated SRBs in Initial Context Setup Request or Bearer Setup Request.

-	Supports indication of SRB creation for EPS bearer ID.

- For non-IP PDN type, supports multiple EPS bearers

eNodeB:

-	Supports creation of SRB(s) as requested by MME in Initial Context Setup Request or Bearer Setup Request.

-	Supports indication of EPS bearer identities also for SRB(s) in RRC connection reconfiguration request.

-	Possible impact related to RLC mapping.

-	Support “RRC Connection Reconfiguration Request” for Control Plane CIoT EPS Optimisation

UE:

-	Indicates the support for dedicated SRB(s) to MME.

-	Supports indication of EPS bearer identities also for SRB(s) in RRC connection reconfiguration request.

-	For non-IP PDN type, support TFTs in Session Management request.

PGW:

-	For non-IP PDN type, supports multiple EPS bearers and indication of traffic type in TFTs in EPS bearer establishment/modification procedures with SGW/MME.

-	Needs to handle IP DL TFTs and map to non-IP EPS bearers e.g. strip out IP protocol headers etc.

PCF/PCRF:

-	For non-IP PDN type, if dynamic PCC is employed, supports multiple PCC rules and indication of traffic type to PGW.

## 6.6	Solution #6: Support establishing dedicated bearer for QoS differentiation for CP based IMS voice service over NB-IoT (GEO)

### 6.6.0	High-level solution Principles

This solution addresses KI#1 to support IMS voice call over NB-IoT via GEO connecting to EPC.

The following principles are applied to this solution:

-	Procedure on Data Transport in Control Plane CIoT EPS Optimisation is used as baseline.

-	EPS enhancement to support dedicated bearer for the IMS voice service over NB-IoT(GEO).

-	IMS signalling is transmitted via the default bearer, IMS voice data is transmitted via the dedicated bearer.

-	Dedicated bearer can be pre-established during IMS PDN connection establishment to reduce delay for the IMS voice service over NB-IoT(GEO).

### 6.6.1	Description

The TS 23.401 [5] specifies dedicated bearer is not supported over NB-IoT. GBR bearer is not supported in the NB-IoT RAT. These restrictions may not enable NB-IoT to support the QoS requirement of an IMS voice service considering the IMS signalling and the IMS voice data transmission have different QoS requirements (i.e. current IMS voice transmission using GBR bearer with QCI = 1, while IMS signalling using non-GBR bearer with QCI = 5)

The solution proposes to remove those restrictions mentioned above to achieve IMS voice service over NB-IoT via GEO connecting to EPC. The solution is based on CP CIoT EPS Optimisation.

To enable the IMS voice transmission via NB-IoT control plane, an IMS PDN connection containing one default bearer and one dedicated bearer is proposed to transmit the IMS voice service, which can be depicted in the following figure:

Figure 6.6.1-1: IMS signalling via default EPS bearer and voice data via a dedicated EPS bearer

The IMS PDN connection based on IP type, therefore, the protocol stack could be as follows:

Figure 6.6.1-2: Protocol stack for IMS signalling

Figure 6.6.1-3: Protocol stack for IMS voice data

Following procedure is an example to create one default bearer and one dedicated bearer to support IMS voice service

### 6.6.2	Procedures

Figure 6.6.2.1: IMS signalling and voice data delivery in CIoT EPS Optimisation over NB-IoT via GEO satellite

Attach

0. A UE, supporting IMS voice service over NB-IoT(GEO), camps on a NB-IoT(GEO) cell.

1. The UE initiates the Attach procedure by the transmission of an Attach Request to the MME, including the Preferred Network Behaviour indicates "Control Plane CIoT EPS Optimisation is preferred", its supporting of IMS voice over NB-IoT indication.

2. The MME process the Attach request from the UE. If the attach is successful, it sends an Attach Accept message to the UE. The MME indicates Control Plane CIoT EPS Optimisation is supported in the Supported Network Behaviour information. If IMS voice service over NB-IoT is supported when the UE is using the NB-IoT(GEO) and the UE is allowed to use IMS voice over NB-IoT(GEO) based on the UE subscription information , the IMS voice over NB-IoT Support indication is contained in the message as well.

If an ESM container is included in the attach request and IMS related APN is provided, an IMS PDN connection is established during the attachment. Otherwise, a separate PDN connection establishment procedure is performed to establish the IMS PDN connection.

PDN connection establishment for IMS voice service

3. Based on the IMS voice over NB-IoT(GEO) Support indication, the UE can initiate a PDN connection establishment for IMS service by using an APN related to the IMS service, and the request PDN type is set as IP.

4. If the IMS PDN connection is allowed to establish, a default bearer for the IMS PDN connection is established.

Moreover, considering the dedicated bearer establishment triggered by IMS call setup will cause additional delays, the dedicated bearer can be pre-established in combination with the default bearer establishment. The NB-IoT (GEO) RAT type is sent to PGW via the create session request. The PGW can make use of this information to grant dedicated bearer to be established for the IMS APN PDN connection. In this case, two EPS bearers (one default bearer identified by EBI-1 and one dedicated bearer identified by EBI-2) are established for the IMS PDN connection over NB-IoT(GEO). To prevent the use of the dedicated bearer before IMS voice session is established, the TFT binding to the dedicated bearer shall be set to block all packets from the UL and DL.

Editor’s note:	Whether new QCI is needed for supporting IMS voice service over NB-IoT NTN via GEO is FFS.

5. The steps 7-12 in Annex F of TS 23.401 [5] are reused.  If both the default bearer and dedicated bearer are activated, the EBI-1 and EBI-2 are provided to the UE.

After the IMS PDN connection is established, the UE can perform IMS registration as specified in clause 5.2.2 of TS 23.228 [6], which is via the default bearer of the IMS PDN connection. Then the UE can initiate IMS voice call setup.

IMS call setup

6. The UE can initiate IMS signalling negotiation with remote UE by using CP CIoT EPS Optimisation as defined in clause 5.3.4B.2. The NAS PDU carries the encrypted uplink IMS signalling and EBI-1 to the eNodeB of the NB-IoT(GEO). The UE maps to NAS PDU carrying the IMS signalling to the SRBx,

6a. the eNodeB of the NB-IoT(GEO), based on configuration, may retrieve the EPS negotiated QoS profile from the MME. The QoS profile may be applied to prioritise between requests from different UEs.

7. The NAS PDU sent in step 6 is relayed to the MME by the eNodeB using a S1-AP Initial UE message.

8. The MME checks the integrity of the incoming NAS PDU and decrypts the data it contains.

9. Based on the received EBI-1, the MME maps the uplink IMS signalling data to the default bearer  and deliveries it to the P-GW.

10. The Downlink IMS signalling is delivered via the default bearer, details can refer to steps 9-12 of Figure 5.3.4B.2-1 of TS 23.401 [5]. The eNB maps the DL traffic carrying the IMS signalling to the SRBx.

11. The  P-CSCF sends diameter AAR to the PCRF which provides the QoS requirement based on the SIP negotiation. The PCRF generates the PCC rule for the IMS voice data and provides it to the P-GW.

12. If the dedicated bearer has already established during the IMS PDN connection establishment, the PCC rule installation may trigger P-GW to initiate dedicated bearer modification as described in clause 5.4.3 of TS 23.401[5] to update the DL TFT. The UL TFT can be updated by the UE locally based on the IMS SDP negotiation result.

Otherwise, if there is no dedicated bearer for the IMS PDN connection, the PCC rule installation may trigger P-GW to initiate the dedicated bearer creation procedure as specified in step 2-11 in clause 5.4.1 of TS 23.401 [5].

13. When the P-GW receives a downlink IMS voice data, it delivers the downlink data to the S-GW and further to MME via the dedicated bearer.

14. The MME encrypts and integrity protects Downlink IMS voice data.

15. The MME sends the encrypt downlink IMS voice data along with EBI-2 using a NAS PDU carried by a Downlink S1-AP message. The eNB maps the DL traffic of IMS voice packet to the SRBy.

16. The NAS PDUwith IMS voice data is delivered to the UE via a Downlink RRC message.

17. The uplink IMS voice data transmission via the dedicated bearer can refer to steps 16-19 of Figure 5.3.4B.3-1 of TS 23.401 [5]. The UE maps to UL traffic of IMS voice packet to the SRBy.

Editor’s note: 	Whether new SRB is needed for IMS signalling or IMS voice data transmission is FFS and depends on RAN implementation.

Editor’s note: 	Whether and how to provide QoS to the IMS voice service via SRB is FFS and depends on RAN implementation.

Editor’s note: 	Whether and how to enhance or optimize IMS registration, IMS call setup procedures to enable IMS voice over NB-IoT NTN via GEO is FFS.

### 6.6.3	Impacts on Services, Entities and Interfaces

UE:

-	Request for PDN connection establishment for IMS voice service when using NB-IoT NTN via GEO

-	Support dedicated bearer when using NB-IoT NTN via GEO for IMS voice service

-	Support to update UL packet filter locally based on the IMS SDP negotiation result.

MME:

-	Support verification of IMS voice call subscription considering NB-IoT (GEO), UE and network capability for supporting IMS voice over NB-IoT(GEO)

-	Provide IMS voice service over NB-IoT support indication to UE

-	Support for PDN connection establishment for IMS voice service when using NB-IoT NTN via GEO

-	Support to establish dedicated bearer,  GBR bearer for UE using IMS service over NB-IoT NTN via GEO

S-GW/P-GW:

-	Support creating the dedicated bearer, GBR bearer for IMS voice service over NB-IoT (GEO).

eNB:

-	Support creating new SRB for IMS signalling or IMS voice data (depends on RAN study)

HSS:

- Subscription data indicating whether IMS voice over NB-IoT (GEO) is allowed

## 6.7	Solution #7: CP based IMS voice call over NB-IoT NTN via GEO satellite using an SRB for both IMS signalling and voice data

### 6.7.0	High-level solution Principles

This solution aims to resolve Key Issue #1, " Support of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC " with the following key principles;

-	The UE negotiates with MME on its capability of voice call support over NB-IoT as well as support of the Control Plane CIoT EPS optimisation and in addition support of S1-U data transfer.

- 	The default EPS bearer is used for both IMS signalling and voice data transmission where an SRB is used for IMS signalling and can be switched to S1-U data transfer for voice data transmission if supported.

-  Bearer modification procedure for the QoS update or resource reservation is skipped or pre-initiated before the call setup in case that voice call would be setup over NB-IoT NTN via GEO satellite.

### 6.7.1	Description

In this proposal, the use of Control plane CIoT EPS Optimisation is assumed to provide IMS voice services via the default EPS bearer. i.e. IMS signalling and IMS voice data are served via the default EPS bearer, which can be updated its non-GBR QoS characteristics if required. Such QoS update is performed before IMS call setup procedure if UE is attached and IMS registered by NB-IoT(GEO). And so the signallings and data are delivered via SRB. Moreover, Control plane CIoT EPS Optimisation can be used initially for SIP signalling for IMS call setup, later if continuous data transmission(e.g. voice call data transmission) is expected, UE may trigger switching to S1-U data transfer.

Editor’s Note: overhead analysis for this approach is to be updated

### 6.7.2	Procedures

#### 6.7.2.1 Attach or TAU procedure negotiating Voice call support over NB-IoT(GEO):

NB-IoT(GEO) cell shall broadcast in the System Information Broadcast, that it supports Control Plane CIoT EPS Optimisation and it can connect to an MME which supports Control Plane CIoT EPS Optimisation.

On camping NB-IoT(GEO), UE who wants to use voice call service over the NB-IoT(GEO), negotiate its capability of voice call support over NB-IoT as well as support of Control Plane CIoT EPS Optimisation by including a Preferred Network Behaviour indication as “Control Plane CIoT EPS Optimisation is supported”. and in addition include “S1-U data transfer is supported”.

Moreover, UE to use voice call service over the NB-IoT(GEO) needs to include the IE "Voice domain preference and UE's usage setting" as the voice domain preference = “IMS PS Voice only” and UE’s usage setting = voice-centric.

For IMS service, an IMS dedicated APN is assumed, which is well-known or pre-configured for voice call over NB-IoT(GEO). So the UE should execute UE requested PDN connectivity procedure for the IMS dedicated APN.

However, due to restriction of NB-IoT RAT that dedicated EPS bearer is not used and GBR bearer is not used, we assume the default EPS bearer is used for both IMS signalling and voice data.

#### 6.7.2.2	Default EPS bearer for both IMS signalling and voice data

For IMS service, we assume the default EPS bearer is used for both IMS signalling and voice data, which can be delivered via an SRB.

Editor’s Note: It is FFS whether an existing SRB or newly defined SRB is used for delivery.

So in case that UE registers IMS as access type as indicating NB-IoT NTN via GEO satellite, if the voice call is setup b/w caller and callee, the resource reservation in EPS for voice call should be handled specifically.

P-CSCF requests the resource reservation for voice call traffic(i.e. GBR traffic) over NB-IoT(GEO) to PCRF, but P-GW considers as it does not allocate dedicated & GBR bearer for the voice call traffic(i.e. skip the resource reservation)  or, if required, performs bearer modification procedure for the default EPS bearer to upgrade its QoS even as a non-GBR bearer.

Editor’s Note: It is FFS whether alternative approach(e.g. P-CSCF provides indication of voice call over NB-IoT(GEO)) is considered.

Moreover if the expected resource for the voice call over NB-IoT(GEO) is static and need upgrade QoS of the default EPS bearer for the voice call traffic, in order to lessen the call setup time such bearer modification procedure can be initiated before IMS call setup procedure.

#### 6.7.2.3	S1-U data transfer activation

If S1-U data transfer is supported, UE triggers switching to data delivery via DRB and S1-U bearer by sending control plane service request with active flag for voice call data transmission, which can be used to differentiate priority of voice data from IMS signalling delivered via an SRB.

Editor’s Note: It is FFS whether or how to differentiate QoS/priority further between IMS signalling and voice data.

### 6.7.3	Impacts on Services, Entities and Interfaces

UE:

- Support and use control plane CIoT EPS optimization and capability of voice call support over NB-IoT.

- Set "Voice domain preference and UE's usage setting" when camping on NB-IoT(GEO).

- Use default EPS bearer for both  IMS signalling and IMS voice data over NB-IoT(GEO) access.

- Use an SRB to deliver IMS signalling and can trigger switching to S1-U bearer to deliver IMS voice data over NB-IoT(GEO) access.

MME:

- Support capability of voice call support over NB-IoT.

- Initiate bearer modification procedure for resource reservation/QoS update for the default EPS bearer before IMS call setup.

P-GW:

- Allocate IMS signalling and voice mapped to the default EPS bearer.

- Ignore the GBR characteristics of IMS voice call data traffic for UE if provided(i.e. skip the bearer modification procedure), or instead perform bearer modification procedure for QoS update for non-GBR characteristics if required

- Initiate bearer modification procedure for resource reservation/QoS update for non-GBR characteristics of the default EPS bearer before IMS call setup.

## 6.8	Solution #8: Combined CP/UP bearers in a PDN connection

### 6.8.0	High-level solution Principles

This solution addresses KI#1, by using one CP bearer and one UP bearer in IMS PDN connection.

This solution utilizes the benefits of both CP and UP for separate transmission of IMS signalling and voice data, by combining a CP bearer and a UP bearer in a IMS PDN connection. The solution only uses one dynamically allocated radio UP bearer for GEO call, so the remaining radio UP bearer (s) can be used for other services during the voice call, if the NB-IoT UE is assumed to support up to 2 UP radio bearers.

### 6.8.1	Description

This solution focus on the establishment of CP and UP bearer for IMS PDN connection. Other aspects for KI#1, e.g. how the UE decides to request IMS PDN connection based on information from EPS, is not described and is assumed to be solved by other solutions.

The CP bearer is the default bearer and established when the IMS PDN connection is setup, by using existing Control Plane CIOT optimization procedure. The CP bearer is used for transmission of IMS signalling. The UP bearer for transmission of voice data is dedicated bearer dynamically established when the voice path needs to be setup, e.g. during the SIP negotiation between two UEs. In this solution User Plane CIOT EPS Optimisation is optional but not preferred, considering it only benefits for occasional data transmission. The UP bearer can be either Non-GBR or GBR bearer which will be aligned with RAN decision.

To support combined CP and UP bearers in IMS PDN connection, the SGW needs to simultaneously maintain corresponding S11-U path and S1-U path during the IMS call.

### 6.8.2	Procedures

#### 6.8.2.1	Network architecture

As specified in Figure 6.8.2.1-1, the internet PDN connection can be established to provide the internet data transfer service and the DRB1 is associated with the default EPS bearer context of the internet PDN connection, thus at most one extra DRB can be established due to non-internet services (NB-IOT only supports 2 DRBs). The UE establishes the IMS PDN connection with default EPS bearer over the SRB (e.g., the SRB1 in Figure 6.8.2.1-1), and when the voice call is initiated the dedicated EPS bearer for voice is established with the DRB2 in Figure 6.8.2.1-1. The RB carries the IMS signalling and the RB carries the voice can use different RLC mode (e.g., IMS signalling over SRB1 is RLC ACK mode, and IMS voice over DRB2 is RLC UNACK mode or RLC TRANSPARENT mode).

Figure 6.8.2.1-1: Bearer mapping for IMS voice over GEO via NB-IoT NTN connected to EPC using CP CIoT

The benefit of this approach is the priority can be configured appropriately, e.g.,: priority of [RRC signalling, NAS signalling, IMS signalling] > priority of IMS voice > priority of Internet data; as illustrated in Table 6.8.2.1-1.

| Traffics | Bearer | Configuration | Priority Level |
| --- | --- | --- | --- |
| RRC signalling, NAS signalling, IMS signalling | SRB1(-bis) (ACK Mode) | SRB by default has higher priority than any DRB | Highest |
| IMS voice | DRB2 (not ACK Mode) | e.g., Priority Level 2 if configured with QCI 1 as specified in TS 23.203 [17] Table 6.1.7-A | Middle |
| Internet data | DRB1 (ACK Mode) | e.g., Priority Level 9 if configured with QCI 10 as specified in TS 23.203 [17] Table 6.1.7-A | Low |

Table 6.8.2.1-1: Example Priority level for different type of traffics

Editor’s note:	Whether a new GBR or non-GBR QCI is needed for the dedicated EPS bearer used for voice, to fit for the characteristics of voice over GEO link is FFS.

Editor’s note:	Whether QCI=1 is sufficient or a new QCI (GBR or non-GBR) value needs to be introduced in order to express the difference between the usual voice media that assumes ptime=20ms and the GEO voice media that assumes different longer ptime value.

#### 6.8.2.2 Combined CP/UP bearers establishment

Figure 6.8.3.2-1: Combined CP/UP bearers establishment

- The UE initiates the PDN Connectivity Request to MME to request IMS PDN connection setup. It may be an initial attach request with PDN connectivity or UE requested PDN connectivity. The UE can indicates its support of combined CP/UP bearers(it is a new capability indication) in Preferred Network Behaviour in initial attach request or TAU.

NOTE: How the UE make the decision to request IMS PDN connection for voice service, e.g. based on information from RAN or EPC, is assumed to be solved by other solutions.

- Based on UE support of combined CP/UP bearers, RAT type(NB-IoT access), UE subscription of using GEO call and network capability, the MME determines to setup the IMS PDU session and allocates S11-U resource for default bearer(CP bearer). The MME sends a Create Session Request to SGW. Before this, the MME selects the SGW supporting combined CP/UP bearers via DNS query.

The MME includes S11-U info(IP address and TEID) and new indicator of combined CP/UP bearers in Create Session Request. MME can include the QoS for default bearer applied for IMS signalling. The exact QoS value(QCI) for CP bearer can be aligned with RAN decision.

During the PDN connection setup procedures, the PGW employs an IP-CAN Session Establishment procedure with the PCRF as defined in TS 23.203 [17] to get the default PCC rules for the UE. The PCRF may configure the QoS parameters (QCI) associated with the default bearer, based on RAT type.

- The SGW returns a Create Session Response to MME, including S11-U info of default bearer, following existing procedure.

- If separation of S11-U from S1-U is required, the SGW shall additionally include the SGW IP address and TEID for S1-U, as described in TS 23.401 [5]. This is different from the S1-U info for dedicated UP bearer applied for subsequent voice data at step 6.

If indicator of combined CP/UP bearers is received in step 2 from MME, the SGW should not change the default bearer from S11-U to S1-U.

- The MME sends Activate Default EPS Bearer request to UE. This message is contained in S1-AP Downlink NAS transport message to eNB. In the NAS message the MME includes combined CP/UP bearers indicator to the UE.

If indicator of combined CP/UP bearers is received, the UE should not change the default bearer from S11-U to S1-U.

Above steps 1-4 follows the existing procedures of PDN Connectivity establishment for Control Plane CIoT EPS Optimisation, with enhancement on the IMS PDN connection to support combined CP/UP bearers.

After the PDN Connectivity is established, the UE uses the CP bearer to transport the SIP signalling to IMS(P-CSCF)

- If a call is received in IMS, following the existing IMS procedures, or the enhanced IMS procedures for NB-IoT NTN access described in other solutions, the IMS(P-CSCF) requests the PCRF to authorize the resources necessary to establish a dedicated bearer for voice data transmission. Then the PCRF follows the Dedicated Bearer Activation Procedure as described in clause 5.4.1 of TS 23.401 [5] by sending to PGW a PCC decision provision (PCC rule with QoS policy) message to modify the IP-CAN session.

- The SGW shall allocate S1-U resource for the bearer if indicator of combined CP/UP bearers is received in step 2 from MME. The SGW should not request creation of additional CP bearer . The SGW sends the Create Bearer Request to MME including the S1-U info for the Dedicated Bearer.

- The MME sends Activate dedicated EPS bearer context request message to UE, if the number of currently activated UP bearers for the UE dose not reach the maximum number of user plane radio bearers(existing NB-IoT specification supports 2 bearers at most). In the message the MME includes QoS(QCI) and TFT for the dedicated UP bearer to the UE.

Activate dedicated EPS bearer context request message is contained in S1-AP Bearer Setup Request message to eNB. The eNB establishes the corresponding UP bearer, based on QoS info in Bearer Setup Request message .

NOTE: The UP bearer can be either Non-GBR or GBR bearer which will be aligned with RAN decision. The exact QCI value will be aligned with RAN decision.

- The MME sends Create Bearer Response to SGW, including EPS Bearer ID and S1-U info for UP bearer from eNB

After the activation of dedicated UP bearer, the UE can transport the voice data over UP bearer based on TFT.

Editor’s note: How to support both CP bearer and UP bearer for a UE accessing NB-IoT needs coordination with RAN.

### 6.8.3	Impacts to Services, Entities and Interfaces

The following impacts are foreseen by this solution:

MME:

- Establishment of combined CP/UP bearers in IMS PDN session, with CP for default bearer and UP for dedicated bearer.

- Indicating to UE and SGW the attribute of combined CP/UP for the IMS PDN connection

SGW:

- Establishment of combined CP/UP bearers in a PDN session, with CP for default bearer and UP for dedicated bearer

PCRF:

- Decides QoS associated with the default CP bearer and dedicated UP bearer in IMS PDN session based on NB-IoT NTN RAT type

UE:

- Establishment of combined CP/UP bearers in a PDN session, with CP for default bearer and UP for dedicated bearer

- The UE should not change the default bearer from S11-U to S1-U, if the IMS PDN connection is CP/UP combined.

eNB:

- Support of activating both CP bearer and UP bearer for a UE, with respective bearer QoS

## 6.9	Solution #9:  Optimisation of IMS voice call initiation over NB-IoT (GEO) when the UE is in Idle

### 6.9.0	High-level solution Principles

This solution addresses KI#1, and the high level principles of the solution are:

-	The UE and Network Support both CP CIoT EPS optimisation and S1-U data transfer.

-	The UE uses IMS user plane for IMS signalling and voice when the UE is in ECM-CONNECTED.

-	The UE indicates active flag in Control Plane Service Request and also includes SIP INVITE in ESM message container when UE the initiate voice call in ECM-IDLE. This activates S1-U bearer and user plane radio bearers for IMS PDN connection.

-	The rest of IMS procedure continues with user plane.

### 6.9.1	Description

The UE triggering IMS voice call over CP have advantage of saving call setup time because Control Plane Service Request message can contain the INVITE message, therefore the waiting time for service request and service accept from the network can be saved. To support QoS of IMS voice data, dedicated GBR bearer is needed over NB-IoT RAT.

Editor's note:	It is FFS whether using GBR bearers provides any benefit for NB-IoT NTN RAT.

This solution describes the enhancement based on the CP CIoT EPS optimisation and establishment of S1-U bearer during Data Transport in CP CIoT EPS Optimisation specified in clause 5.3.4B.4 of TS 23.401 [5]. Basically, the UE uses user plane default bearer for IMS signalling and dedicated bearer for IMS voice when the UE is in ECM-CONNECTED. But when the UE is in ECM-IDLE, the UE use the Control Plan Service Request to send SIP INVITE message with indicating active flag when initiating voice call. The MME forwards the SIP INVITE encapsulated in the ESM message to SGW via S11-U interface and establishes S1-U bearer. Then the rest of IMS SIP signalling and voice are transferred via user plane default and dedicated bearers as existing procedure.

NOTE:	Since the NB-IoT UE only support maximum 2 data radio bearers, the MME need to set Control Plane Only Indicator for all other PDNs except for IMS PDN.

After the voice call end the eNB may trigger the S1 release procedure (e.g. due to user inactivity), the MME releases UE context to the eNB and release the radio bearers.

Since the proposed solution only optimize the call initiation part when the UE in ECM-IDLE, it can be used together with other solutions.

NOTE:	This solution only addresses the optimisation for MO voice call case, for MT voice call case, when the UE is idle, the UE performs Network Triggered Service Request procedure in 5.3.4.3 of 23.401 [5] to use user plane for IMS voice and signalling.

### 6.9.2	Procedures

Figure 6.9.3-1: High-level procedure for IMS voice over NB-IoT (GEO)

1.	UE provides Attach Request to the NB-IoT (GEO) satellite including Voice domain preference indicates IMS PS Voice only and UE's usage setting indicates voice centric, the Preferred Network Behaviour indicates both Control Plane CIoT EPS Optimisation and S1-U data transfer are supported. The UE may use IMS PDN as attach PDN.

The MME proceeds the Attach request from the UE. If the attach is successful, it sends an Attach Accept message to the UE. The MME indicates both Control Plane CIoT EPS Optimisation and S1-U data transfer are supported in the Supported Network Behaviour information. The MME also indicates IMS voice over NB-IoT Support indication.

If the UE is not attached using the IMS PDN, the UE establishes the IMS PDN Connection.

When the UE is not initiating a MO voice call, it basically uses the user plane default bearer for IMS signalling. For IMS voice, it always uses the user plane dedicated bearer.

Editor's note:	It is FFS how the authorization of the UE to establish dedicated bearer is done.

2.	After Attach or PDN Connection procedure, if the UE in ECM-CONNECTED performs IMS registration procedure over the default bearer of IMS PDN Connection (user plane). If the UE in ECM-IDLE triggers UE triggered Service Request in 5.3.4.1 of 23.401 [5] to use user plane because this is not voice call initiation from the UE.

3.	The eNB may trigger S1 release procedure (e.g. due to user inactivity) and the RRC Connection is released.

4.	When the UE initiates a voice call, it sends SIP INVITE message over the Control Plane using CP CIoT EPS Optimisation to the MME. The UE includes the SIP INVITE in the ESM DATA TRANSPORT message of ESM message container in the Control Plane Service Request and indicates active flag towards the MME.

Editor's note:	Including ESM message container in the Control Plane Service Request and indicates active flag needs coordination with CT1.

5.	If the S11-U connection is not established, the MME sends and receive Modify Bearer Request/Response with SGW.

6.	The MME forwards the uplink data (SIP INVITE message) from the UE through S11-U bearer.

7.	After the MME forwards the uplink data, the MME sends a Release Access Bearers Request message to the Serving GW that requests the release of all S11-U bearers for the UE.

Editor's Note:	It is FFS how to ensure that the MME triggers Release Access Bearers Request after sending the INVITE message.

If downlink packets arrive for the UE, the Serving GW starts buffering downlink packets received for the UE until the step 11.

8-10.	Based on the Control Plane Service Request with active flag, the MME sends S1-AP Initial Context Setup Request message to the eNodeB for the IMS PDN connection to activate user plane radio bearers and S1-U bearers.

11.	The MME sends a Modify Bearer Request message to setup S1-U User Plane.

As the user plane radio bearer is setup, the UE uses user plane bearer to transfer IMS signalling data..

12.	SIP 100 Trying signalling is transferred over the default EPS bearer (UP)

13-15.	P-CSCF sends to PCRF the AA-Request to activate a dedicated bearer for voice media. Dedicated bearer activation procedure specified in clause 5.4.1 of TS 23.401 [5] is performed, and the PCRF responds to P-CSCF with the AA-Answer.

16.	P-CSCF forwards the SIP 183 progress over the default EPS bearer (UP).

17.	The procedures for IMS voice call setup continues over the default EPS bearer (UP).

18.	IMS voice media is transferred over the dedicated GBR EPS bearer (UP).

19.	The voice call end.

20.	The eNB may trigger S1 release procedure (e.g. due to user inactivity) and the RRC Connection is released.

### 6.9.3	Impacts on Services, Entities and Interfaces

UE:

-	Support dedicated bearer over NB-IoT access for GBR.

-	Indicates active flag and includes ESM message container to MME using Control Plane Service Request when voice call start in ECM-IDLE.

-	Support of IMS SIP INVITE data transport in control plane.

eNB:

-	Support dedicated bearer over NB-IoT (GEO) access for GBR.

MME:

-	Indicate to the UE the support of IMS voice over NB-IoT.

-	Support dedicated bearer over NB-IoT (GEO) access for GBR.

PGW:

-	Support dedicated bearer over NB-IoT (GEO) access for GBR.

## 6.10	Solution #10: Support of CP/UP combination and QoS for IMS voice service over GEO satellite with a pair of PDN connections

### 6.10.0	High-level solution Principles

This solution addresses KI#1, in particular QoS aspects.

This solution utilizes a pair of PDN connections.

NOTE:	This solution does not address any optimization aspect e.g., on how to achieve a shorter call setup time or how to transfer SIP message/voice media efficiently in CP or UP. This solution is intended to be used together with other solutions to achieve such optimization.

### 6.10.1	Description

Principles of this solution are as follows:

- No impacts on the QoS mechanism in UE using NB IoT.

Editor's note: FFS whether there is still no impact on QoS when the use of GBR is required.

- Two PDN connections are used for IMS voice, where one without DRB is used for SIP message exchange and the other with DRB is used for voice media. These two PDN connections are linked together by using (newly introduced) PDN connectivity pair ID.

- Each of the two PDN connections, when established with DRB, uses a default bearer only as per the existing specification. Different QCI is used for the default bearer depending on APN.

Editor's note: Whether to use GBR for the default bearer or not and what impact is there are FFS.

- UE may explicitly request to establish PDN connection without DRB by using (newly introduced) "Control Plane Only Indicator requested" indicator.

- UE internally associates contexts of the two PDN connections.

- PCRF internally associates contexts of the two PDN connections.

Proposed solution for the issue of coexistence of S11-U and S1-U is as follows:

- Consider that UE has established an SGi PDN connection using S1-U and an SGi PDN connection using S11-U and that the UE has become in the idle mode. When the UE has pending data for either of these SGi PDN connections, the UE triggers the RRC Connection Resume procedure that is used to resume the SGi PDN connection using S1-U. Upon reception of UE Context Resume Request, MME sends to S-GW both S1-U eNB TEID, which MME has received in Initial Context Setup response and stored, and S11-U MME TEID, which MME reserves in itself, by using Modify Bearer Request. Then both these SGi PDN connections are activated. UE sends the pending data via the corresponding PDN connection. In order to allow UE to establish an SGi PDN connection using S1-U and an SGi PDN connection using S11-U, MME is configured to allow establishing these SGi PDN connections under a certain condition (e.g., UE has sent Preferred CIoT network behaviour and UE's usage setting=voice in the Attach procedure).

Aspects for further consideration are as follows:

- Whether QCI=1 is sufficient or a new QCI value needs to be introduced in order to express the difference between the usual voice media that assumes ptime=20ms and the GEO voice media that assumes ptime= 80ms or 100ms.

- Whether and how to perform authorization check.

Editor's note: FFS whether and how to perform authorization check.

- Potential impacts to IMS nodes (e.g., P-CSCF) due to the use of two PDN connections (i.e., two UE IP addresses).

### 6.10.2	Procedures

#### 6.10.2.1	General

The procedures in the below are an example and depict the case where SIP messages are transferred in CP and voice media is transferred in UP.

#### 6.10.2.2	Establishment of PDN connection for SIP message exchange

Figure 6.10.2.2-1: Establishment of PDN connection for SIP message exchange

- UE performs Attach without PDN Connectivity. The Subscription-Data that MME has received from HSS contains APN-Configuration-Profile for APN=ims and QCI=5 and for APN=ims_media (a new value) and QCI=1 (or a new QCI value for the GEO voice media).

- UE that uses NB IoT recognizes that two PDN connections are required for the GEO voice call; one for SIP message exchange and the other for voice media. UE may decide to use Control Plane for SIP message exchange, while still having available DRB(s). UE sends the PDN connectivity request containing APN=ims, "Control Plane Only Indicator requested" (a new indicator), and PDN connectivity pair ID (a new parameter).

- Based on "Control Plane Only Indicator requested", MME decides to set Control Plane Only PDN Connection Indication. MME sends to S-GW the Create Session Request containing APN=ims, Control Plane Only PDN Connection Indication, and PDN connectivity pair ID.

- S-GW sends to P-GW the Create Session Request containing APN=ims, Control Plane Only PDN Connection Indication, and PDN connectivity pair ID.

- P-GW sends to PCRF the CC-Request containing APN=ims, "Control Plane Only Indicator" (a new indicator), and PDN connectivity pair ID. PCRF recognizes that this PDN connection is for SIP message exchange only and not for voice media.

- The rest is the same as the existing procedure.

- PDN connection is established. This PDN connection uses NAS for transferring data (i.e., SIP messages) between UE and MME and uses U-plane between MME and P-GW via S-GW.

#### 6.10.2.3	IMS voice call setup

##### 6.10.2.3.1	MO procedure

NOTE:	Steps 9 to 14 in the below for establishing a PDN connection for voice media can be performed any time after a PDN connection for SIP messages are established as per the procedure in 6.10.3.2 to shorten the call setup time.

Figure 6.10.2.3.1-1: IMS voice call setup MO

- A path for SIP message exchange is established as a result of the procedure depicted in clause 6.10.3.2.

- UE sends SIP INVITE to MME by using NAS.

- MME forwards SIP INVITE towards P-CSCF on U-plane via S-GW and P-GW.

- P-CSCF sends SIP INVITE to S-CSCF.

- P-CSCF receives SIP 183 Session Progress.

- P-CSCF sends to PCRF the AA-Request to establish a dedicated bearer for voice media.

- PCRF recognizes that the targeted PDN connection is for SIP message exchange only and that UE is expected to establish another PDN connection for voice media when the UE receives SIP 183 Session Progress. PCRF decides not to send any message to P-GW. PCRF sends to P-CSCF the AA-Answer with success. (new behavior of PCRF)

- P-CSCF sends SIP 183 Session Progress. SIP 183 Session Progress is transferred to MME on U-plane via P-GW and S-GW.

- MME sends SIP 183 Session Progress to UE by using NAS.

- UE recognizes the need to establish PDN connection for voice media. UE sends the PDN connectivity request containing APN=ims_media and PDN connectivity pair ID. UE sets the value of the PDN connectivity pair ID to be the same as that for the PDN connection for SIP message exchange.

- MME recognizes the value of the PDN connectivity pair ID is the same as that for the PDN connection for SIP message exchange and selects P-GW that is the same as that for the PDN connection for SIP message exchange. MME confirms that UE has available DRB(s). MME sends to S-GW the Create Session Request containing APN=ims_media, QCI=1(or a new QCI value for the GEO voice media), and PDN connectivity pair ID.

- S-GW sends to P-GW the Create Session Request containing APN=ims_media, QCI=1(or a new QCI value for the GEO voice media), and PDN connectivity pair ID.

- P-GW recognizes the value of the PDN connectivity pair ID is the same as that for the PDN connection for SIP message exchange and selects PCRF that is the same as that for the PDN connection for SIP message exchange. P-GW sends to PCRF the CC-Request containing APN=ims_media, QCI=1(or a new QCI value for the GEO voice media), and PDN connectivity pair ID. PCRF internally associates contexts of the two PDN connections having the same PDN connectivity pair ID. PCRF recognizes that this newly established PDN connection is for voice media.

- PCRF sends to P-GW the CC-Answer.

- The procedure for PDN connectivity establishment continues.

- PDN connection is established. This PDN connection uses U-plane for transferring data (i.e., voice media) between UE and P-GW via S-GW.

- The procedure for IMS voice call setup continues.

- S-CSCF sends SIP 200 OK to P-CSCF.

- When the procedure of Enabling of IP Flows is used, P-CSCF sends the AA-Request to PCRF.

- PCRF receives the AA-Request to the PDN connection for SIP message exchange, checks the context for the PDN connection for voice media as well, and decides to send the RA request to P-GW for PDN connection for voice media. PCRF sends to P-CSCF the AA-Answer with success. (new behavior of PCRF)

- PCRF sends the RA-Request to P-GW for the PDN connection for voice media.

- P-GW sends the RA-Answer to PCRF.

- The procedure for IMS voice call setup continues. Then, IMS voice call is setup.

##### 6.10.2.3.2	MT procedure

NOTE:	Steps 9 and 10 in the below for establishing a PDN connection for voice media can be performed any time after a PDN connection for SIP messages are established as per the procedure in 6.10.3.2 to shorten the call setup time.

Figure 6.10.2.3.2-1: IMS voice call setup MT

0.	A path for SIP message exchange is established as a result of the procedure depicted in clause 6.10.3.2.

1.	P-CSCF receives SIP INVITE.

2.	P-CSCF sends SIP INVITE. SIP INVITE is transferred to MME on U-plane via P-GW and S-GW.

3.	MME sends SIP INVITE to UE by using NAS.

4.	UE sends SIP 183 Session Progress to MME by using NAS.

5.	MME forwards SIP 183 Session Progress towards P-CSCF on U-plane via S-GW and P-GW.

6.	P-CSCF sends SIP 183 Session Progress to S-CSCF.

7.	P-CSCF sends to PCRF the AA-Request to establish a dedicated bearer for voice media.

8.	PCRF recognizes that the targeted PDN connection is for SIP message exchange only and that UE is expected to establish another PDN connection for voice media when the UE receives SIP 183 Session Progress. PCRF decides not to send any message to P-GW. PCRF sends to P-CSCF the AA-Answer with success. (new behavior of PCRF)

9.	Upon reception of SIP INVITE in step 3, UE recognizes the need to establish PDN connection for voice media. UE sends the PDN connectivity request containing APN=ims_media and PDN connectivity pair ID. UE sets the value of the PDN connectivity pair ID to be the same as that for the PDN connection for SIP message exchange.

10.	The procedure for PDN connectivity establishment continues.

11.	PDN connection is established. This PDN connection uses U-plane for transferring data (i.e., voice media) between UE and P-GW via S-GW.

12.	The procedure for IMS voice call setup continues. Then, IMS voice call is setup.

### 6.10.3	Impacts on Services, Entities and Interfaces

UE:

- Handling of a new parameter "PDN connectivity pair ID" and of a new indicator "Control Plane Only Indicator requested".

- In order to solve the issue of coexistence of S11-U and S1-U:

  - Handling of the order of activation of an SGi PDN connection using S1-U and activation of an SGi PDN connection using S11-U.

MME:

- Selection of the same P-GW for two PDN connections that have the same value for a new parameter "PDN connectivity pair ID".

- Handling of a new indicator "Control Plane Only Indicator requested".

- In order to solve the issue of coexistence of S11-U and S1-U:

  - Making sure that S-GW receives all the necessary S1-U eNB TEID and S11-U MME TEID when SGi PDN connections are activated.

  - Allowing coexistence of an SGi PDN connection using S1-U and an SGi PDN connection using S11-U under certain conditions.

S-GW:

- Transparent forwarding of a new parameter "PDN connectivity pair ID"

P-GW:

- Selection of the same PCRF for two PDN connections that have the same value for a new parameter "PDN connectivity pair ID"

PCRF:

- Handling of a new parameter "PDN connectivity pair ID" and of a new indicator "Control Plane Only Indicator"

Editor's note: Potential impacts to IMS nodes (e.g., P-CSCF) due to the use of two PDN connections (i.e., two UE IP addresses) are FFS.

Editor's note: The impacts on P-CSCF is FFS.

## 6.11	Solution #11: Support of PDN connection where packets are transported as Non-IP over the GEO satellite and seen as IP by IMS nodes

### 6.11.0	High-level solution Principles

This solution addresses KI#1, in particular message size optimization.

In this solution, for UL, UE sends Non-IP packets and P-GW adds IP header. For DL, P-GW receives IP packets, removes IP header, and sends Non-IP packets.

This solution is intended to be used together with another solution shown in "Support of CP/UP combination and QoS for IMS voice service over GEO satellite with a pair of PDN connections".

### 6.11.1	Description

Principles of this solution are as follows:

- For UL, UE sends Non-IP packets and P-GW adds IP header. For DL, P-GW receives IP packets, removes IP header, and sends Non-IP packets.

- UE receives an IP address allocated to it from P-GW at establishment of the PDN connection for voice media and uses the IP address when creating contents of SDP messages.

- UE sends to P-GW an IP address allocated in IMS AGW, when the UE receives the IP address in SDP. P-GW uses this IP address when adding IP header to voice media packets.

- P-GW is assumed to know the IP address of P-CSCF by pre-configuration. P-GW uses this IP address when adding IP header to SIP messages.

Editor's note: The current description mentions IP header only. FFS whether to consider UDP/TCP header addition/removal as well. FFS what impact is required if UDP/TCP header addition/removal is also treated.

### 6.11.2	Procedures

#### 6.11.2.1	General

The procedures in the below are an example and depict the case where the proposed solution is used together with another solution shown in "Support of CP/UP combination and QoS for IMS voice service over GEO satellite with a pair of PDN connections". This other solution is referred to as the base solution in the below.

#### 6.11.2.2	Establishment of PDN connection for SIP message exchange

Figure 6.11.2.2-1: Establishment of PDN connection for SIP message exchange

Steps of Figure 6.11.2.2-1 of the base solution are applied with the following modifications.

- This step is the same as step 0 of the base solution.

- This step is the same as step 1 of the base solution with the following modifications: The PDN connectivity request additionally contains PDN type=IPv6 and Extended protocol configuration options that have "CN header handling request" (a new indicator).

- This step is the same as step 2 of the base solution with the following modifications: The Create Session Request additionally contains PDN type=IPv6 and Extended protocol configuration options that have "CN header handling request" (a new indicator).

- This step is the same as step 3 of the base solution with the following modifications: The Create Session Request additionally contains PDN type=IPv6 and Extended protocol configuration options that include "CN header handling request" (a new indicator).

- This step is the same as step 4 of the base solution.

Steps 5 to 9 correspond to step 5 of the base solution.

- PCRF sends to P-GW the CC-Answer. P-GW recognizes the need of the CN header handling. P-GW receives or assigns an IP address for UE and recognizes that this IP address is to be used for the source address for UL packets for the CN header handling for this PDN connection. P-GW recognizes that an P-CSCF address that the P-GW has known by configuration is to be used for the destination address for UL packets for the CN header handling for this PDN connection based on Access Point Name = ims and PDN connectivity pair ID. (new behavior of P-GW)

- P-GW sends to S-GW the Create Session Response containing PDN Address Allocation that includes the allocated IPv6 address and Extended protocol configuration options that include "CN header handling accepted indication" (a new indicator).

- S-GW sends to MME the Create Session Response containing Control plane only indication, PDN Address Allocation that includes the allocated IPv6 address, and Extended protocol configuration options that include "CN header handling accepted indication" (a new indicator).

- MME sends to UE the Activate Default EPS Bearer Context Request containing Control plane only indication, PDN Address Allocation that includes the allocated IPv6 address, and Extended protocol configuration options that include "CN header handling accepted indication" (a new indicator). UE recognizes the IPv6 address allocated to it.

- UE sends to MME the Activate Default EPS Bearer Context Accept.

- This step is the same as step 6 of the base solution with the following modifications: This PDN connection is set to transfer Non-IP packets between UE and P-GW and the P-GW is expected to add IP header for UL packets and remove IP header for DL packets.

#### 6.11.2.3	IMS registration

Figure 6.11.2.3-1: IMS registration

- A path for SIP message exchange is established as a result of the procedure depicted in clause 6.11.3.2.

- UE sends SIP Register without IP header to MME by using NAS.

- MME sends SIP Register without IP header to P-GW on U-plane via S-GW.

- P-GW adds IP header to SIP Register without IP header; P-GW adds the UE IP address as the source address and the P-CSCF IP address as the destination address. (new behavior of P-GW)

- P-GW sends SIP Register with IP header to P-CSCF.

- P-CSCF sends SIP Register to S-CSCF.

- P-CSCF receives 200 OK.

- P-CSCF sends 200 OK to P-GW.

- P-GW removes IP header of 200 OK. (new behavior of P-GW)

- P-GW sends 200 OK without IP header to MME on U-plane via S-GW.

- MME sends 200 OK without IP header to UE by using NAS.

#### 6.11.2.4	IMS voice call setup

##### 6.11.2.4.1	MO procedure

NOTE:	Steps 13 to 18 in the below for establishing a PDN connection for voice media can be performed only after UE receives SDP answer (in MO side)

Editor's note: FFS whether SIP timers are negatively impacted.

Figure 6.11.2.4.1-1: IMS voice call setup MO

Steps of Figure 6.11.2.3.1-1 of the base solution are applied with the following modifications.

- This step is the same as step 0 of the base solution.

- This step is the same as step 1 of the base solution with the following modifications: SIP INVITE is without IP header.

Steps 2 to 4 correspond to step 2 of the base solution.

- MME sends SIP INVITE without IP header to P-GW on U-plane via S-GW.

- P-GW adds IP header to SIP INVITE without IP header; P-GW adds the UE IP address as the source address and the P-CSCF IP address as the destination address. (new behavior of P-GW)

- P-GW sends SIP INVITE with IP header to P-CSCF.

Steps 5 to 8 are the same as steps 3 to 6 of the base solution.

Steps 9 to 11 correspond to step 7 of the base solution.

- P-CSCF sends SIP 183 Session Progress to P-GW.

- P-GW removes IP header of SIP 183 Session Progress. (new behavior of P-GW)

- P-GW sends SIP 183 Session Progress without IP header to MME on U-plane via S-GW.

- This step is the same as step 8 of the base solution with the following modifications: SIP 183 Session Progress is without IP header.

- This step is the same as step 9 of the base solution with the following modifications: UE recognizes an IP address allocated in IMS AGW in SDP answer in SIP 183 Session Progress without IP header. The PDN connectivity request additionally contains "Destination Address for CN header handling" (a new parameter) that includes the IP address allocated in IMS AGW, PDN type=IPv6, and Extended protocol configuration options that have "CN header handling request" (a new indicator).

- This step is the same as step 10 of the base solution with the following modifications: The Create Session Request additionally contains "Destination Address for CN header handling" (a new parameter) that includes the IP address allocated in IMS AGW, PDN type=IPv6, and Extended protocol configuration options that have "CN header handling request" (a new indicator).

- This step is the same as step 11 of the base solution with the following modifications: The Create Session Request additionally contains "Destination Address for CN header handling" (a new parameter) that includes the IP address allocated in IMS AGW, PDN type=IPv6, and Extended protocol configuration options that have "CN header handling request" (a new indicator).

- This step is the same as step 12 of the base solution.

- This step is the same as step 13 of the base solution with the following modifications: P-GW recognizes the need of the CN header handling. P-GW receives or assigns an IP address for UE and recognizes that this IP address is to be used for the source address for UL packets for the CN header handling for this PDN connection. P-GW recognizes that an IMS AGW address that the P-GW has received in "Destination Address for CN header handling" (a new parameter) is to be used for the destination address for UL packets for the CN header handling for this PDN connection based on Access Point Name = ims_media and PDN connectivity pair ID. (new behavior of P-GW)

- This step is the same as step 14 of the base solution.

- This step is the same as step 15 of the base solution with the following modifications: The PDN connection is set to transfer Non-IP packets between UE and P-GW and the P-GW is expected to add IP header for UL packets and remove IP header for DL packets.

- This step is the same as step 16 of the base solution with the following modifications: Exchanged SIP messages are without IP header between UE and P-GW.

Steps 21 to 25 are the same as steps 17 to 21 of the base solution.

- This step is the same as step 22 of the base solution with the following modifications: Exchanged SIP messages are without IP header between UE and P-GW.

- UE sends UL voice media packets without IP header to P-GW on U-plane via S-GW.

- P-GW adds IP header to UL voice media packets without IP header; P-GW adds the UE IP address as the source address and the IMS AGW IP address as the destination address. (new behavior of P-GW)

- P-GW sends voice media packets with IP header to IMS AGW.

- P-GW receives voice media packets with IP header from IMS AGW.

- P-GW removes IP header of voice media packets. (new behavior of P-GW)

- P-GW sends voice media packets without IP header to UE on U-plane via S-GW.

##### 6.11.2.4.2	MT procedure

NOTE:	Steps 13 to 14 in the below for establishing a PDN connection for voice media can be performed only after UE receives SDP offer (in MT side).

Editor's note: FFS whether SIP timers are negatively impacted.

Figure 6.11.2.4.2-1: IMS voice call setup MT

Steps of Figure 6.11.2.3.2-1 of the base solution are applied with the following modifications.

Steps 0 to 1 are the same as steps 0 to 1 of the base solution.

Steps 2 to 4 correspond to step 2 of the base solution.

- P-CSCF sends SIP INVITE to P-GW.

- P-GW removes IP header of SIP INVITE. (new behavior of P-GW)

- P-GW sends SIP INVITE without IP header to MME on U-plane via S-GW.

- This step is the same as step 3 of the base solution with the following modifications: SIP INVITE is without IP header.

- This step is the same as step 4 of the base solution with the following modifications: SIP 183 Session Progress is without IP header.

Steps 7 to 9 correspond to step 5 of the base solution.

- MME sends SIP 183 Session Progress without IP header to P-GW on U-plane via S-GW.

- P-GW adds IP header to SIP 183 Session Progress without IP header; P-GW adds the UE IP address as the source address and the P-CSCF IP address as the destination address. (new behavior of P-GW)

- P-GW sends SIP 183 Session Progress with IP header to P-CSCF.

Steps 10 to 12 are the same as step 6 to 8 of the base solution.

- This step is the same as step 9 of the base solution with the following modifications: UE recognizes an IP address allocated in IMS AGW in SDP offer in SIP INVITE without IP header. The PDN connectivity request additionally contains "Destination Address for CN header handling" (a new parameter) that includes the IP address allocated in IMS AGW, PDN type=IPv6, and Extended protocol configuration options that have "CN header handling request" (a new indicator).

- This step is the same as step 10 of the base solution.

- This step is the same as step 11 of the base solution with the following modifications: The PDN connection is set to transfer Non-IP packets between UE and P-GW and the P-GW is expected to add IP header for UL packets and remove IP header for DL packets.

- This step is the same as step 12 of the base solution with the following modifications: Exchanged SIP messages are without IP header between UE and P-GW.

- UE sends UL voice media packets without IP header to P-GW on U-plane via S-GW.

- P-GW adds IP header to UL voice media packets without IP header; P-GW adds the UE IP address as the source address and the IMS AGW IP address as the destination address. (new behavior of P-GW)

- P-GW sends voice media packets with IP header to IMS AGW.

- P-GW receives voice media packets with IP header from IMS AGW.

- P-GW removes IP header of voice media packets. (new behavior of P-GW)

- P-GW sends voice media packets without IP header to UE on U-plane via S-GW.

### 6.11.3	Impacts on Services, Entities and Interfaces

Since the solution is an extension of the solution in 6.10 "Support of CP/UP combination and QoS for IMS voice service over GEO satellite with a pair of PDN connections", the impacts to UE, MME, S-GW, P-GW, PCRF, and P-CSCF are also inherited from that solution. In addition, the following impacts apply.

UE:

- Handling of new indicators "CN header handling request" and "CN header handling accepted indication".

- Extracting an IMS AGW IP address from SDP and informing it to MME by using a new parameter "Destination Address for CN header handling".

Editor's note: It is FFS how the UE generates TCP/UDP payload without IP header.

MME:

- Transparent forwarding of a new parameter "Destination Address for CN header handling"

S-GW:

- Transparent forwarding of a new parameter "Destination Address for CN header handling"

P-GW:

- Handling of new indicators "CN header handling request" and "CN header handling accepted indication”

- Adding IP header for UL packets and removing IP header from DL packets.

- Receiving an IP address in a new parameter "Destination Address for CN header handling" from S-GW and using it when adding IP header for UL packets as the destination address.

Editor's note: The impacts on P-CSCF is FFS.

## 6.12	Solution #12: Reduce SIP message size and number during INVITE procedure, P-CSCF ascts as B2BUA

### 6.12.0	High-level solution Principles

This solution address Key Issue #2: "IMS Enhancements for GEO NB-IoT NTN Access".

In the context of NB-IoT (GEO) voice communication scenarios, to improve IMS signaling efficiency, the following optimizations are proposed in this solution:

‌1. Signaling Reduction via Precondition Disabling‌

- Disable the precondition feature to eliminate precondition-related message exchanges.

‌2. Minimalist Message Composition‌

- Mandate that UEs and P-CSCFs include ‌only essential headers and parameters‌ (c.f. Annex C) sent to each other during INVITE procedures complying with RFC 3261 [18] and RFC 4566 [19].

3. ‌Context-Aware Information Reuse‌

- P-CSCF caches UE-specific data obtained during the IMS REGISTER phase to eliminate redundant data transmission in subsequent INVITE messages.

- Prevent duplicate information across different part of the SIP message (c.f. Annex C).

4. ‌Compression Enhancement‌

- Deploy signalling compression (SigComp) as specified in RFC 5049 [20] on the UE and P-CSCF.

### 6.12.1	Description

The solution presumes an EPS bearer for voice data is not pre-established, which will be addressed by resolutions for Key Issue #1, and proposes P-CSCF acts as B2BUA.

### 6.12.2	Procedures

#### 6.12.2.1	Registration

During the IMS registration procedure, the P-CSCF stores information received from UE. During the INVITE procedure, the UE does not need to provide that information again, and P-CSCF can provide that information towards terminating side according to the stored information, which can save message size between UE and P-CSCF during INVITE procedure.

Following is an example SIP REGISTER request message sent from UE to P-CSCF:

REGISTER tel:+8613512345678 SIP/2.0

v:SIP/2.0/UDP pc33.vs.ims.mno.3g.org:5060;branch=z9hG4bKnashd9

f:<sip:+8613512345678@vs.ims.mno.3g.org>;tag=192830

t:<sip:A>;tag=201563

i:a84b

CSeq:32 REGISTER

Max-Forwards:70

m:<sip:+8613512345678@vs.ims.mno.3g.org>,q=0.7;expires=3600

P-Access-Network-Info:3GPP-NB-IoT(GEO);utran-cell-id-3gpp=C359A

Authorization:Digest username="13512345678",realm="vs.ims.mno.3g.org",uri="sip:+8613512345678@vs.ims.mno.3g.org",response="a3f549b13f477562f4445b7277cd83c1",nonce="qlwqVapVqlWqVapVqlWqVUUQA5HEt9VVZ3t1TM221cg=",algorithm=AKAv1-MD5

If the P-Access-Network-Info (PANI) indicates that the RAT type of the UE is NBIoT (GEO), the P-CSCF stores information in Via (v), From (f), Contact (m), and PANI header field.

During the registration procedure, the IMS AS is aware of the UE's access network information according to the PANI header field.

#### 6.12.2.2	Mobile originating

The following figure depicts the mobile originating IMS INVITE procedure in the NB-IoT (GEO) scenario.

In this procedure, the UE does not send information that already sent during IMS registration procedure in clause 6.12.2.1, and does not provide duplicated information in To header field that already provided in Request-URI. The P-CSCF replaces the information in Via (v), From (f), and Contact (m) header field as well as insert PANI header field according to the stored information, and replaces information in To (t) header field according to the Request-URI.

Figure 6.12.2.2-1: Mobile originating

0.	The UE has registered in IMS as described in clause 6.12.2.1. The user dials number to initiate a voice call.

1.	The UE may perform Service Request procedure to transit from RRC-IDLE state to RRC-CONNECTED state. EPS bearer for voice data is not pre-established during the Service Request procedure.

2.	If RAT type is NBIoT (GEO), the UE sends SIP INVITE with SDP offer that only indicates supported codecs for GEO voice to the P-CSCF, wherein the information already provided in Request-URI is not provided in To header field and information already provided during registration procedure is not included.

INVITE tel:+8613587654321 SIP/2.0

v:SIP/2.0/UDP A;branch=z9hG4bK122456

f:<sip:A>;tag=205847

t:<sip:A>;tag=147302

m:<sip:A>

i:a9gH

CSeq:81 INVITE

Max-Forwards:70

c:application/sdp

l:120

v=0

o=- 4567 1234 IN IP4 1.1.1.1

s=-

c=IN IP4 192.168.1.100

m=audio 49155 RTP/AVP 0 8

t=0 0

a=rtpmap:0 PCMU/6000

a=rtpmap:8 PCMA/6000

The P-CSCF sends SIP 100 to the UE.

SIP/2.0 100

v:SIP/2.0/UDP A;branch=z9hG4bK122456

f:<sip:A>;tag=205847

t:<sip:A>;tag=147302

i:a9gH

CSeq:81 INVITE

Editor’s note: Whether SIP 100 between UE and P-CSCF can be omitted is FFS.

3.	If, during the IMS registration procedure, the RAT type indicated in the PANI header field is NBIoT (GEO), the P-CSCF instructs the AGW to allocate audio resource for network side as well as allocate and configure audio resource for UE side according to the SDP offer.

4.	The P-CSCF restores information according to stored information during registration procedure and Request-URI of the INVITE message, and forwards the modified SIP INVITE with SDP offer to IMS core. The SDP offer includes audio resource information allocated by AGW for network side. According to the NBIoT (GEO) RAT type in PANI header field during IMS registration procedure, the P-CSCF acts as B2BUA to add indication of supporting 100rel and precondition in the modified SIP INVITE request.

INVITE tel:+8613587654321 SIP/2.0

v: SIP/2.0/UDP p-cscf.vs.ims.mno.3g.org:5060; branch=z9hG4bK886431UKf234t

v: SIP/2.0/UDP pc33.vs.ims.mno.3g.org:5060; branch=z9hG4bK901butKNw56ub

f: <sip:+8613512345678@vs.ims.mno.3g.org>; tag=a2Xwhg47uT90

t: <sip: tel:+8613587654321>; tag=702Krb82gIkv

m: <sip:+8613512345678@vs.ims.mno.3g.org>

i: 9byT8XUwhL5k

CSeq: 81962507 INVITE

Max-Forwards: 69

P-Access-Network-Info: 3GPP-NB-IoT(GEO); utran-cell-id-3gpp=C359A

Support: 100rel, precondition

c: application/sdp

l: 345

v=0

o=- 94567123 89561264 IN IP4 10.47.16.5

s=-

c=IN IP4 192.168.2.100

m=audio 51893 RTP/AVP 0 8

t=0 0

a=rtpmap:0 PCMU/6000

a=rtpmap:8 PCMA/6000

a=cur:qoslocal none

a=cur:qosremote none

a=des:qos mandatory local sendrecv

a=des:qos none remote sendrecv

5.	After step 3, the P-CSCF uses the resource information for UE side to trigger the EPS procedure for bearer activation and configuration (e.g., TFT configuration in PGW).

NOTE 1:	This step is addressed by solution for Key Issue #1.

6-8.	The P-CSCF receives SIP 183 Progressing with SDP answer from IMS core. The P-CSCF delegates UE to send SIP PRACK to IMS core, and receives SIP 200 OK (PRACK) from IMS core.

9-10.	The P-CSCF sends SIP UPDATE with SDP offer to IMS core, which is after receiving notification from PCRF that the bearer resource is ready, then receives SIP 200 OK (UPDATE) from IMS core.

11.	The P-CSCF receives SIP 180 Ringing from IMS core. The P-CSCF sends SIP 180 to the UE.

SIP/2.0 180

v:SIP/2.0/UDP A;branch=z9hG4bK122456

f:<sip:A>;tag=205847

t:<sip:A>;tag=147302

i:a9gH

CSeq:81 INVITE

When the UE receives the SIP 180, it plays ring tone.

12.	The P-CSCF receives SIP 200 OK from IMS core. The P-CSCF sends SIP 200 with SDP answer to the UE, and the SDP answer includes the audio resource information allocated by the AGW for UE side.

SIP/2.0 200

v:SIP/2.0/UDP A;branch=z9hG4bK122456

f:<sip:A>;tag=205847

t:<sip:A>;tag=147302

m:<tel:+8613587654321>

i:a9gH

CSeq:81 INVITE

c:application/sdp

l:98

v=0

o=- 4567 1234 IN IP4 1.1.1.1

s=-

c=IN IP4 192.168.2.100

m=audio 93524 RTP/AVP 0

t=0 0

a=rtpmap:0 PCMU/6000

a=ptime:80

13.	The UE sends SIP ACK to the P-CSCF, wherein the information already provided in Request-URI is not provided in To header field and information already provided during registration procedure is not included.

ACK tel:+8613512345678 SIP/2.0

v:SIP/2.0/UDP A;branch=z9hG4bK654321

f:<sip:A>;tag=205847

t:<sip:A>;tag=147302

i:a9gH

CSeq:54 ACK

Max-Forwards:70

14.	The P-CSCF restores information according to stored information during registration procedure and Request-URI of the ACK message, and forwards the modified SIP ACK to IMS core.

Editor’s note: It is FFS how to handle the race condition if voice packet goes faster than SIP 200 OK with SDP answer and cause the delay of receiving the SDP answer by the originating UE for sending out voice data.

#### 6.12.2.3	Mobile terminating

The following figure depicts the mobile terminating IMS INVITE procedure in the NB-IoT (GEO) scenario.

In this procedure, the UE does not send information that already sent during IMS registration procedure in clause 6.12.3.1, and does not provide duplicated information in To header field that already provided in Request-URI. The P-CSCF replaces the information in Via (v), From (f), and Contact (m) header field as well as insert PANI header field according to the stored information, and replaces information in To (t) header field according to the Request-URI.

Figure 6.12.2.3-1: Mobile terminating

1.	The P-CSCF receives SIP INVITE with SDP offer from IMS core, which may indicate support of precondition.

2.	The P-CSCF instructs the AGW to allocate audio resource for UE side.

3.	If RAT type is NBIoT (GEO), the P-CSCF sends a SIP INVITE without precondition and with SDP offer that only indicates supported codecs for GEO voice to the UE, wherein the information already provided in Request-URI is not provided in To header field. The SDP offer includes resource information allocated by the AGW for UE side.

INVITE tel:+8613512345678 SIP/2.0

v:SIP/2.0/UDP p-cscf.vs.ims.mno.3g.org:5060;branch=z9hG4bK122456

f:<tel:+8613587654321>;tag=205847

t:<sip:A>;tag=147302

m:<sip:+8613587654321@vs.ims.mno.3g.org>

i:a9gH

CSeq:81 INVITE

Max-Forwards:70

c:application/sdp

l:120

v=0

o=- 4567 1234 IN IP4 1.1.1.1

s=-

c=IN IP4 192.168.2.100

m=audio 93524 RTP/AVP 0 8

t=0 0

a=rtpmap:0 PCMU/6000

a=rtpmap:8 PCMA/6000

The SIP INVITE may trigger EPS to perform network triggered service request procedure towards the UE, and EPS bearer for voice data is not pre-established during the procedure.

4.	The UE sends SIP 183 with SDP answer to the P-CSCF.

SIP/2.0 183

v:SIP/2.0/UDP p-cscf.vs.ims.mno.3g.org:5060;branch=z9hG4bK122456

f:<tel:+8613587654321>;tag=205847

t:<sip:A>;tag=147302

i:a9gH

CSeq:81 INVITE

c:application/sdp

l:98

v=0

o=- 4567 1234 IN IP4 1.1.1.1

s=-

c=IN IP4 192.168.2.100

m=audio 49155 RTP/AVP 0

t=0 0

a=rtpmap:0 PCMU/6000

5.	The P-CSCF instructs the AGW to configure resources for UE side according to the SDP answer, as well as allocate and configure resources for network side.

6.	The P-CSCF uses the resource information for UE side to trigger the EPS procedure for bearer activation and configuration (e.g., TFT configuration in PGW).

NOTE:	This step is addressed by solution for Key Issue #1.

7.	The sends SIP 183 Progressing to IMS core. If the received SIP INVITE request indicates support of 100rel and precondition, the P-CSCF acts as B2BUA to add indication of requiring 100rel and precondition in the SIP 183 response.

8.	If the P-CSCF receives SIP PRACK from IMS core, it sends SIP 200 OK (PRACK) to IMS core.

9.	If the P-CSCF receives SIP UPDATE from IMS core, it sends SIP 200 OK (UPDATE) to IMS core, which is after receiving notification from PCRF that the bearer resource is ready.

10.	The UE sends SIP 180 to the P-CSCF and alerts the user.

SIP/2.0 180

v:SIP/2.0/UDP p-cscf.vs.ims.mno.3g.org:5060;branch=z9hG4bK122456

f:<tel:+8613587654321>;tag=205847

t:<sip:A>;tag=147302

i:a9gH

CSeq:81 INVITE

The P-CSCF sends SIP 180 Ringing to IMS core.

11.	The user answers the call, the UE sends SIP 200 to the P-CSCF.

SIP/2.0 200

v:SIP/2.0/UDP p-cscf.vs.ims.mno.3g.org:5060;branch=z9hG4bK122456

f:<tel:+8613587654321>;tag=205847

t:<sip:A>;tag=147302

i:a9gH

CSeq:81 INVITE

m:<sip:A>

The P-CSCF sends SIP 200 OK to IMS core, wherein the Contact header field is set according to the stored Contact header field during IMS registration procedure. If the P-CSCF has not send SDP answer to IMS core, it includes SDP answer in the SIP 200 OK towards IMS core.

12.	The P-CSCF receives SIP ACK from IMS core. The P-CSCF sends SIP ACK to the UE.

ACK tel:+8613512345678 SIP/2.0

v:SIP/2.0/UDP p-cscf.vs.ims.mno.3g.org:5060;branch=z9hG4bK098765

f:<tel:+8613587654321>;tag=205847

t:<sip:A>;tag=147302

i:a9gH

CSeq:17 ACK

Max-Forwards:70

#### 6.12.2.4	Information for call setup time evaluation

The following table shows the estimated message information during mobile originating procedure:

| Sequence | Message Name | Message Size (B) (proportional) | Transmission Delay (s) in 1kbps |
| --- | --- | --- | --- |
| 1 | INVITE (UL) | 335 (+52) | 3.10 |
| 2 | 100 Trying (DL) | 117 (+52) | 1.35 |
| 3 | RRCConnectionReconfiguration (DL) | 92 (+5) | 0.78 |
| 4 | RRCConnectionReconfigurationComplete (UL) | 2 (+5) | 0.06 |
| 5 | 180 Ringing (DL) | 117 (+52) | 1.35 |

The following table shows the estimated message information during mobile terminating procedure:

| Sequence | Message Name | Message Size (B) (proportional) | Transmission Delay (s) in 1kbps |
| --- | --- | --- | --- |
| 1 | INVITE (DL) | 373 (+52) | 3.40 |
| 2 | 100 Trying (UL) | 148 (+52) | 1.60 |
| 3 | 183 Progressing (UL) | 273 (+52) | 2.60 |
| 4 | RRCConnectionReconfiguration (DL) | 92 (+5) | 0.78 |
| 5 | RRCConnectionReconfigurationComplete (UL) | 2 (+5) | 0.06 |
| 6 | 180 Ringing (UL) | 148 (+52) | 1.60 |

NOTE:	The SIP message size is original before applying sigcomp.

### 6.12.3	Impacts on Services, Entities and Interfaces

Editor's note:	Further impact is FFS.

UE:

-	Support not providing duplicated information in a SIP message that was already sent during registration procedure or is already provided in another place of the SIP message.

-	Support sigcomp.

-	Support not indicating precondition during INVITE procedure.

P-CSCF:

-	Support storing values of header fields during registration procedure.

-	Support replacing information in a SIP message according to the stored information during registration procedure and/or if already provided in another place of the SIP message.

-	Support B2BUA.

-	Support sigcomp.

# 7	Interim Agreements

## 7.1	Agreed Principles for KI#5 of UE-SAT-UE communication via UPF only onboard satellite for non-IMS services

To supports KI#5 of UE-SAT-UE communication via UPF only onboard satellite for non-IMS services, the following principles are concluded:

Principle 1: The existing R18 mechanism (Local switch for UE-to-UE communications via UPF deployed on GEO satellite) can be reused to trigger UE-SAT-UE communication, select UPF(s) onboard satellite(s) and establish UE-SAT-UE communication via UPF(s) only onboard NGSO satellite(s).

Principle 2: The following existing 5G VN features apply for KI#5

- A single SMF or a single SMF set is supported.

- The UEs are in the same 5G VN group.

- Local switch within UPF on-board satellite, N6 and N19 tunnel between UPFs on-board different satellites are supported.

- Both IP PDU Session type and Ethernet PDU Session type are supported.

- Only unicast UE-SAT-UE communication is supported.

Principle 3: The SMF determines whether to continue or to fall back the UE-SAT-UE communication due to serving satellite change, based on the information about the availability of ISL(s) between satellites serving the UEs as well as the 5G VN group supporting UE-SAT-UE communication. How SMF gets the information is per implementation, e.g., based on UPF report or from transport network via a proprietary interface.

Principle 4: PSA UPF on ground and UL CL/BP/local PSA UPF onboard satellite is supported as baseline. To support service continuity, the following apply:

- For the UE-SAT-UE communication continuity case, the SMF reuses existing procedures for Simultaneous change of Branching Point or UL CL and additional PSA for a PDU Session as described in clause 4.3.5.7 of TS 23.502 [3] for the UE.

- For the ground fallback case the SMF coordinates existing procedures at both UEs for Removal of additional PDU Session Anchor and Branching Point or UL CL as described in clause 4.3.5.5 of TS 23.502 [3]. The SMF firstly removes UL CL/L-UPF of peer UE and configure PDR&FAR and N6/N19 routing in ground PSA UPF(s) (if two UEs are using different PSA UPFs) during the satellite handover of local UE, then removes UL CL/L-UPF of local UE after satellite handover of local UE.

Principle 5: For PSA UPF on satellite, SSC mode 2 or 3 can be used to support service continuity. It is assumed that all the 5G VN group members are accessing NTN (no DN connectivity while UE-Sat-UE with PSA UPF).

# 8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-04 | SA2#168 | S2-2504185 |  |  |  | TR 23.700-19: FS_5GSAT_Ph4_ARC TR Skeleton | 0.0.0 |
| 2025-04 | SA2#168 | S2-2504186 |  |  |  | TR 23.700-19: FS_5GSAT_Ph4_ARC TR Scope | 0.1.0 |
| 2025-04 | SA2#168 | S2-2504187 |  |  |  | TR 23.700-19: Architecture principles and assumptions | 0.1.0 |
| 2025-04 | SA2#168 | S2-2504188 |  |  |  | TR 23.700-19: Architecture assumptions | 0.1.0 |
| 2025-04 | SA2#168 | S2-2504223 |  |  |  | Key issues (KIs) for WT#1 | 0.1.0 |
| 2025-05 | SA2#169 | S2-2505964 |  |  |  | Solution #1: UP based IMS voice call over NB-IoT NTN via GEO satellite using a default EPS bearer for both IMS signalling and voice data | 0.2.0 |
| 2025-05 | SA2#169 | S2-2505965 |  |  |  | Solution #2: Supporting establishing EPS bearers for IMS voice via user plane with IP type PDN connection | 0.2.0 |
| 2025-05 | SA2#169 | S2-2505767 |  |  |  | Solution #3: IP Type IMS PDN connection over User Plane with NIDD voice transfer between UE and PGW | 0.2.0 |
| 2025-05 | SA2#169 | S2-2505966 |  |  |  | Solution#4: Support of IMS Voice Call over NB-IoT NTN via GEO by using User Plane NIDD | 0.2.0 |
| 2025-05 | SA2#169 | S2-2505771 |  |  |  | Solution#5: Support of IMS voice NB-IoT NTN connected to EPC using control plane bearers and new SRBs | 0.2.0 |
| 2025-05 | SA2#169 | S2-2505967 |  |  |  | Solution #6: Support establishing dedicated bearer for QoS differentiation for CP based IMS voice service over NB-IoT (GEO) | 0.2.0 |
| 2025-05 | SA2#169 | S2-2505968 |  |  |  | Solution #7: CP based IMS voice call over NB-IoT NTN via GEO satellite using an SRB for both IMS signalling and voice data | 0.2.0 |
| 2025-05 | SA2#169 | S2-2505773 |  |  |  | Solution #8: Combined CP/UP bearers in a PDN connection | 0.2.0 |
| 2025-05 | SA2#169 | S2-2505774 |  |  |  | Solution #9: Optimisation of IMS voice call initiation over NB-IoT (GEO) when the UE is in Idle | 0.2.0 |
| 2025-05 | SA2#169 | S2-2505775 |  |  |  | Solution #10: Support of CP/UP combination and QoS for IMS voice service over GEO satellite with a pair of PDN connections | 0.2.0 |
| 2025-05 | SA2#169 | S2-2505969 |  |  |  | Solution #11: Support of PDN connection where packets are transported as Non-IP over the GEO satellite and seen as IP by IMS nodes | 0.2.0 |
| 2025-05 | SA2#169 | S2-2505535 |  |  |  | Annex B: Methodology for estimating call setup time | 0.2.0 |
| 2025-05 | SA2#169 | S2-2505536 |  |  |  | Solution #12: Reduce SIP message size and number during INVITE procedure, P-CSCF ascts as B2BUA; and Annex C: Mandatory parameters for SIP and SDP | 0.2.0 |
| 2025-05 | SA2#169 | S2-2505777 |  |  |  | Agreed principles for KI#5 of UE-SAT-UE communication via UPF only onboard satellite for non-IMS services; and Key Issue #5: UE-SAT-UE communication via UPF only onboard satellite for non-IMS services | 0.2.0 |
| 2025-05 | SA2#169 |  |  |  |  |  | 0.2.0 |
| 2025-05 | SA2#169 |  |  |  |  |  | 0.2.0 |
| 2025-05 | SA2#169 |  |  |  |  |  | 0.2.0 |

Annex B:
Methodology for estimating call setup time

## B.1	Formula for estimated call setup time calculation

### B.1.1	General

To ensure consistency in evaluating solutions based on estimated call setup time, solutions for evaluation must adhere to a standardized baseline methodology‌ for calculating the estimated call setup time. This requirement enables direct comparison of latency performance across competing solutions through quantitative analysis.

The following is the example formula for calculation of estimated call setup time (ECST) from satellite to terrestrial, based on ideal radio circumstance (e.g., no retrying and retransmission, no segmentation, etc.) and no simultaneous transmission for multiple signals:

ECSTsatToTerrs≈ETstateTransition                               [MO]s+ETimsDelay                   [MO]s+ETepsDelay                   [MO]s+ETterr[s]

The following is the example formula for calculation of estimated call setup time (ECST) from satellite to satellite based on the same condition as above:

ECSTsatToSats≈ETstateTransition                               [MO]s+ETstateTransition                               [MT]s+ETimsDelay                   [MO]s+ETimsDelay                   [MT]s+ETepsDelay                   [MO]s+ETepsDelay                   [MT]s

Wherein, ETstateTransition is the estimated time of state transition from RRC-IDLE to RRC-CONNECTED (including paging for MT); ETimsDelay is the estimated delay of IMS procedure for call setup; ETepsDelay is the estimated delay of EPS procedure for call setup. The parameter ETterr is the transmission and propgation delay considering terristrial side, half of VoLTE (4-5s based on experience), 2.5s is assumed.

### B.1.2	Estimated time of IMS procedure for call setup

The following is the formula for calculation of estimated time for IMS signalling exchange:

ETimsDelays≈SsignallingOfIMSbits+SimsHeaderbitsRtransmissionbpss+Dpropagation×3×NUL-IMS+NDL-IMS[s]

NOTE 1:	It needs to be multiplied by 3 for sending an UL signalling considering 2 additional signalings for uplink resource scheduling.

NOTE 2:	For solution addressing Key Issue #2, ETimsDelay can be used for evaluation.

Wherein, variable SsignallingOfIMS is the size of IMS signalling, e.g., SIP or I1 message; variable SimsHeader is the size of protocol overhead (typically UDP/IP/PDCP/RLC/MAC) that carrying IMS signalling, typical value is 52 Bytes (UDP/IPv6 used); variable Rtransmission is the transmission rate; variable NUL-IMS and NDL-IMS are the number of UL and DL IMS messages respectively.

The parameter Dpropagation is 0.28s according to clause 7.4.1 of TS 22.261 [8].

### B.1.3	Estimated time of EPS procedure during call setup

ETepsDelays≈SsignallingOfEPSbits+SepsHeaderbitsRtransmissionbpss+Dpropagation×3×NUL-EPS+NDL-EPS[s]

Wherein, variable SsignallingOfEPS is the size of EPS signalling, e.g., RRC signalling for bearer resource reservation; variable SepsHeader is the size of protocol overhead (i.e., PDCP/RLC/MAC) that carrying EPS signalling; variable NUL-EPS and NDL-EPS are the number of UL and DL EPS messages respectively.

### B.1.4	Estimated time of state transition

Editor’s note: The formula for calcuating estimated time for state transition from RRC-IDLE to RRC-CONNECTED and estimated time for paging MT UE is FFS.

## B.2	Example message size and corresponding transmission delay

Following figure shows the typical IMS mobile call processing in TN scenario:

Figure B.2-1: IMS mobile call processing in TN scenario

Following table shows the example SIP signaling size and corresponding transmission delay:

Table B.2-1: Example SIP message size and corresponding transmission delay in 1kbps

| Sequence | Message Name | Message Size (B) (proportional) | Transmission Delay (s) in 1kbps |
| --- | --- | --- | --- |
| 1 | INVITE | 2,564 (+52) | 20.93 |
| 2 | 100 Trying | 445 (+52) | 3.98 |
| 3 | 183 Progressing | 1,705 (+52) | 14.06 |
| 4 | PRACK | 1,054 (+52) | 8.85 |
| 5 | 200 OK (PRACK) | 726 (+52) | 6.22 |
| 6 | UPDATE | 1,929 (+52) | 15.85 |
| 7 | 200 OK (UPDATE) | 1,494 (+52) | 12.37 |
| 8 | 180 Ringing | 983 (+52) | 8.28 |
| 9 | 200 OK (INVITE) | 1,365 (+52) | 11.34 |
| 10 | ACK | 840 (+52) | 7.14 |

Following table shows the example EPS signaling size and corresponding transmission delay during the call setup:

Table B.2-2: Example EPS message size and corresponding transmission delay in 1kbps

| Sequence | Message Name | Direction | Message Size (B) (proportional) | Transmission Delay (s) in 1kbps |
| --- | --- | --- | --- | --- |
| a.1 | RRCConnectionReconfiguration | DL | 92 (+5) | 0.78 |
| a.2 | RRCConnectionReconfigurationComplete | UL | 2 (+5) | 0.06 |

Annex C:
Mandatory parameters for SIP and SDP

According to RFC 3261 [18] clause 20 and 20.1, following header fields are mandatory for SIP messages considering UDP transport protocol is used:

| Header field | where | ACK | BYE | CAN | INV | OPT | REG |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Call-ID (i) | c(1) | m | m | m | m | m | m |
| Contact (m) | R | o | - | - | m | o | o |
| Contact (m) | 2xx | - | - | - | m | o | o |
| Content-Type (c) (NOTE) |  | * | * | - | * | * | * |
| CSeq | c(1) | m | m | m | m | m | m |
| From (f) | c(1) | m | m | m | m | m | m |
| Max-Forwards | R | m | m | m | m | m | m |
| To (t) | c(2) | m | m | m | m | m | m |
| Via (v) | R | m | m | m | m | m | m |
| Via (v) | rc(1) | m | m | m | m | m | m |
| (1): header field is copied from the request to the response. (2): copied from the request to the response with possible addition of tag. |  |  |  |  |  |  |  |

Some header fields (e.g., Allow, Supported) are recommended to be included in SIP messages related to call setup (e.g., INVITE request, 2xx response), but in voice over NBIoT (GEO) scenario, the minimal parameters of those header fields can be required to avoid negotiation of them.

The Content-Length header field is required when stream-based protocol (such as TCP) is used as a transport.

According to RFC 4566 [19], following type lines are mandatory for SDP:

v=  (protocol version)

o=  (originator and session identifier)

s=  (session name)

c=  (connection information)

t=  (time the session is active)

m=  (media name and transport address)

a=* (zero or more media attribute lines)

NOTE:	Although "a=" line is optional, but it is necessary for audio media type. The "c=" line is included in all media or at session level.

According to RFC 3261 [18]: "initial Request-URI of the message SHOULD be set to the value of the URI in the To field", generally the value of To header field is duplicated with the value of Request-URI.