| 3GPP TR 23.700-21 V0.3.0 (2024-04) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Application enablement architecture for mobile metaverse services; (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	9

3.1	Terms	9

3.2	Symbols	9

3.3	Abbreviations	9

4	Key issues	9

4.1	Key issue #1: Enabler support for managing spatial anchors	9

4.1.1	Description	9

4.1.2	Open issues	10

4.2	Key Issue #2: Exposure of user sensitive information	10

4.2.1	Description	10

4.2.2	Open issues	10

4.3 	Key issue #3: Digital avatars support	11

4.3.1	Description	11

4.3.2	Open issues	11

4.4 	Key issue #4: Spatial mapping	12

4.4.1	Description	12

4.4.2	Open issues	12

4.5	Key issue #5: Support for avatar discovery and QoS control	12

4.5.1	Description	12

4.5.2	Open issues	13

4.6	Key issue #6: Support device discovery to offload task for metaverse services	13

4.6.1	Description	13

4.6.2	Open issues	14

4.7	Key issue #7: Support for metaverse services requiring multiple devices	14

4.7.1	Description	14

4.7.2	Open issues	14

4.8	Key issue #8: Enabler support for spatial anchors based services	14

4.8.1	Description	14

4.8.2	Open issues	15

4.x	Key issue #x: <Title>	15

4.x.1	Description	15

4.x.2	Open issues	15

5	Application enablement architecture requirements	15

5.1	General requirements	15

5.x	<application enabler layer capability x> requirements	15

6	Application enablement architecture for metaverse services	15

6.1	Option #1: On-network mobile metaverse application layer architecture	15

6.1.1	Application enablement architecture	15

6.1.2	Functional Elements	16

6.1.2.1	Mobile Metaverse Enablement Server	16

6.1.2.2	Mobile Metaverse Enablement Client	16

6.1.3	Reference Points	16

6.1.3.1	MM-UU	16

6.1.3.2	MM-S	17

6.1.3.3	MM-E	17

6.1.3.4	MM-C	17

6.2	Option #2: A-DACM architecture to support Metaverse services	17

6.2.1	Application enablement architecture	17

6.2.2	Functional Elements	18

6.2.2.1	A-DACM function	18

6.2.3	Reference Points	18

6.2.3.1	SEAL-S	18

6.2.4	Service-based interface	18

6.2.4.1	Sdacm	18

6.x	Option #x: <title>	19

6.x.1	Application enablement architecture	19

6.x.2	Functional Elements	19

6.x.3	Reference Points	19

7	Solutions	19

7.0	Mapping of solutions to key issues	19

7.1	Solution #1: Spatial anchor discovery	19

7.1.1	Solution description	19

7.1.1.1	General	19

7.1.1.2	Procedure	20

7.1.2	Architecture Impacts	21

7.1.3	Corresponding APIs	21

7.1.4	Solution evaluation	21

7.2	Solution #2: Support for spatial anchor management	21

7.2.1	Solution description	21

7.2.2	Architecture Impacts	22

7.2.3	Procedures	22

7.2.3.1	Creating spatial anchor	22

7.2.3.2	Updating spatial anchor	22

7.2.3.3	Get spatial anchor	23

7.2.3.4	Spatial anchor information subscription	23

7.2.4	Solution evaluation	24

7.3	Solution #3: Spatial anchor subscription	24

7.3.1	Solution description	24

7.3.1.1	General	24

7.3.1.2	VAL server subscription procedure for information about UE with spatial anchor interest	25

7.3.1.3	VAL client subscription procedure for spatial anchors in range	26

7.3.2	Architecture Impacts	27

7.3.3	Corresponding APIs	27

7.3.4	Solution evaluation	27

7.4	Solution #4: Support for spatial anchor analytics information	27

7.4.1	Solution description	27

7.4.2	Procedures	27

7.4.2.1	Spatial anchor analytics information (subscription-notify)	27

7.5	Solution #5: Support for digital avatars	28

7.5.1	Solution description	28

7.5.2	Architecture Impacts	29

7.5.3	Procedures	29

7.5.3.1	Create application specific avatar profile	29

7.5.3.2	Update application specific avatar profile	30

7.5.3.3	Get application specific avatar profile	31

7.5.3.4	Delete application specific avatar profile	32

7.5.3.5	Application specific avatar profile(s) subscription	32

7.5.4	Solution evaluation	33

7.7	Solution #7: Digital avatar support by A-DACM	36

7.7.1	Solution description	36

7.7.1.1	General	36

7.7.1.2	Avatar profile creation	36

7.7.1.3	Avatar profile upload	37

7.7.1.4	Avatar linking to user/subscriber	37

7.7.1.5	Avatar download	38

7.7.2	Architecture Impacts	38

7.7.3	Corresponding APIs	38

7.7.4	Solution evaluation	39

7.8	Solution #8: Support for spatial map management	39

7.8.1	Solution description	39

7.8.2	Architecture Impacts	39

7.8.3	Procedures	39

7.8.3.1	Producing spatial map	39

7.8.3.2	Update spatial map	40

7.8.3.3	Get spatial map	40

7.8.3.4	Subscribe/unsubscribe spatial map	41

7.8.4	Solution evaluation	42

7.x	Solution #x: <title>	42

7.x.1	Solution description	42

7.x.2	Architecture Impacts	42

7.x.3	Corresponding APIs	42

7.x.4	Solution evaluation	42

8	Deployment scenarios	42

8.1	General	42

8.x	Deployment model #x: <Title>	42

9	Business Relationships	42

10	Overall evaluation	42

10.1	Architecture evaluations	42

10.2	Key issue evaluations	43

11	Conclusions	43

11.1	General conclusions	43

11.2	Conclusions of key issue #x	43

Annex A (informative): Change history	44

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document is a technical report capturing the study on application enablement architecture for mobile metaverse services over 3GPP networks. The aspects of the study include defining key issues and their related solutions to identify architecture requirements, application layer functional model (either by defining new functional model or by enhancing existing functional model) to support identified architectural requirements and corresponding solutions to provide enablement layer support to metaverse applications.

The study is based on the requirements as defined in 3GPP TS 22.156 [2] and related use cases in 3GPP TR 22.856 [3]. The study takes into consideration the work done in other 3GPP working groups (like SA2, SA4).

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.156: "Mobile Metaverse Services; Stage 1".

[3]	3GPP TR 22.856: "Feasibility Study on Localized Mobile Metaverse Services".

[4]	3GPP TR 23.700-77: "Study on system architecture for next generation real time communication services; Phase 2".

[5]	3GPP TR 23.700-70: "Study on architecture enhancement for Extended Reality and Media service (XRM); Phase 2".

[6]	3GPP TR 26.813: "Study of Avatars in Real-Time Communication Services".

[7]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[8]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[9]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[10]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[11]	3GPP TR 26.119: "Media Capabilities for Augmented Reality".

[12]	3GPP TS 23.542: "Application layer support for Personal IoT Network".

[13]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Avatar profile: It is avatar specific configuration and parameters (e.g. allowed locations) applicable to one or more application(s). The avatar profile may be associated with one or more VAL user(s).

Application-Digital Asset Container Management: An entity that securely manages the application related digital assets (e.g. avatars).

For the purposes of the present document, the following terms given in 3GPP TS 22.156 [2] apply:

Avatar

Digital asset

Digital wallet

Spatial anchor

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

MDRB					Metaverse Digital Representation Block

MMEC	Mobile Metaverse Enablement Client

MMEL					Mobile Metaverse Enablement  Layer

MMES					Mobile Metaverse Enablement  Server

A-DACM	Application - Digital Asset Container Management

# 4	Key issues

## 4.1	Key issue #1: Enabler support for managing spatial anchors

### 4.1.1	Description

SA1 in 3GPP TS 22.156 [2] defines spatial anchors as "an association between a location in space (three dimensions) and service information that can be used to identify and access services, e.g. information to access AR media content".

Application providers can use spatial anchors to associate application content and services with location information which can be managed by the 3GPP system. In turn, spatial anchors can be used by application clients to discover and access application content and services in a location aware fashion.

Clause 5.4 "Use Case on Spatial Anchor Enabler" of 3GPP TR 22.856 [3] provides detailed use case for spatial anchor enabler. The use case defines mainly two entities:

1)	Spatial anchor producer: It creates the spatial anchor along with its place/location in the 3D space. It determines what to share and its location, and any constraints (e.g. who to share the spatial anchor with) and additional information.

2)	Spatial anchor consumer: It recognizes anchors associated with locations in 3D space, and use the spatial anchor to obtain the associated information.

In order to create spatial anchor, the producer captures the location of the product and assocates a new spatial anchor with this location and product information. The producer can adjust/update the spatial anchors for its location or service information or removes it completely. The consumer can retrieve information about spatial anchors.

SA6 already supports SEAL layer. SA6 can enhance the existing SEAL capability (like location management) to enable support for spatial anchor.

### 4.1.2	Open issues

SA6 defined enablers as of now do not support operations required for managing spatial anchors. The open issues are:

1)	How to enhance existing appliction enabler (either adding new capability in existing server or defining new capability server) to support CRUD operations for spatial anchors (association between location and service information on customer premises)?

2)	How to discover spatial anchors by the consumer (e.g. UE, VAL server)?

3)	How can UE(s) within a certain range of spatial anchor be detected?

## 4.2	Key Issue #2: Exposure of user sensitive information

### 4.2.1	Description

Ensuring appropriate user consent has been obtained is a critical aspect when handling sensitive information relating to or collected from a user, their devices or the applications installed at their devices. This is highlighted at stage 1 where many of the requirements captured in TS 22.156 [2] are subject to operator policy, regulatory requirements and user consent, the latter being of particular importance to this key issue.

For instance, with the expected capability to access, manage and expose user specific avatar related information through the enabler layer it is of utmost importance to capture the consent of the user. Aspects of such information might also be obtained through core network capability exposure, e.g., relating to user identity, body movement or location, and re-exposed by the enabler layer but not without the consent of the user. Another example are, digital assets, where ownership rights are considered to be classified as sensitive information and the owner's consent must be obtained before other users or third party(service provider) can use them. An example of a digital asset is a user designed and produced digital human image. When another user likes that digital human image and wants to use it, they need to request and obtain the owner's consent before doing so.

Each user needs to be in full control of which network and device entities are allowed to access, manage and expose information pertaining to themselves and their devices. Such end user approvals ensure that only legitimate, user trusted, entitles can obtain and utilise information and services involving that user's sensitive information.

### 4.2.2	Open issues

SA6 defined enablers as of now do not consider usage of user consent in the context of supporting localized mobile metaverse services. The open issues are:

-	How to ensure appropriate usage of user consent at the enabler layer to support localized mobile metaverse services in 3GPP specified networks? Appropriate usage of user consent includes ensuring only the legitimate use by non-owners of user sensitive information such as digital assets.

NOTE 1:	Aspects pertaining to the definition of end-user consent/authorization over APIs are in the scope of SA3.

NOTE 2:	Obtaining end-user consent is expected to leverage the RNAA framework, where it is of note that resource owner communication with the authorization function in the CAPIF core function was not in scope of Rel-18.

NOTE 3:	Some aspects may depend on interactions with SA WG2 FS_UIA_ARC. For example, utilisation of user-specific identities in relation to specific components of a user’s information.

NOTE 4：The solution of the case of authorized use of user sensitive information by non-owners may need coordination with SA3.

Editor's Note: The open issues of this key issue can be revisited to consider mobile metaverse constructs such as digital avatars.

Editor's Note: The solution to the key issue needs coordination with SA3.

## 4.3 	Key issue #3: Digital avatars support

### 4.3.1	Description

In addition to new 5GS performance requirements, support for Metaverse services have introduced new use cases and requirements related to optimal support for multi-user, highly performant applications. These use cases led to new CN functionality supporting richer digital representations of physical entities (e.g. user profiles in SA2, XR scene in SA4)

As defined in 3GPP TS 22.156 [2], Avatars are digital representations of users interacting with the metaverse and with other users. The application enabler layer can enable creation, discovery, and management of avatar profiles for users to offload applications and enable Core Network functionality across services and verticals.

NOTE 1: The work in SA6 focuses on providing enablement service to application specific Avatar profile and if available, can leverage the work done in SA2 and SA4.

Editor's Note: How application specific Avatar profile can be linked to user subscription is FFS and depends on SA2 work.

Therefore, solutions addressing this key issue may also leverage work from SA2 and SA4 on user profiles and digital assets.

The following use cases can be considered for study of value-added features:

-	A user provides configuration (or preferences) indicating which metaverse application services can access the avatar information when the user performs transition between metaverse services, to make sure that consistency and continuity of their digital representation is maintained.

-	SA6 can maintain a list of applications allowed to use the avatar (similar to allowed MNO list for federation in EDGEAPP)

-	A User creates multiple avatars and indicates his/her preferences such as:  use of specific avatars based on location,  use of audio-only service only when experiencing low bandwidth.

-	SA6 can maintain user’s choices and: provide avatar information to different application servers based on the location of the user, reject requests for avatar-based communication in poor network conditions, etc.

### 4.3.2	Open issues

This key issue will study:

-	What SA6-defined information is required to expose the SA2-defined, SA4-defined and SA3-defined avatar information to support allowed application configuration, location-based avatar contextual information?

-	Whether and how application enablement layer manages and exposes information about digital avatars  to the consumers (i.e. application clients and servers) or across verticals?

NOTE:	The study considers non-IMS based metaverse services.

## 4.4 	Key issue #4: Spatial mapping

### 4.4.1	Description

In clause 5.2.1 "Localized mobile metaverse service" of 3GPP TS 22.156 [2],

Localized mobile metaverse services are immersive and integrated into a user's ordinary experiences. Such service experiences are location-related and can include presentation of AR, MR media.

Users' localization is important in order to discover spatial anchors. The 5G system offers a spatial localization service to determine this information.

[R-5.2.1-003] Subject to operator policy, regulatory requirements and user consent, the 5G system shall provide a means for a UE to provide sensor data, (e.g., from UE sensors, cameras, etc.) to the network in order to derive localization information, e.g., to produce or modify a spatial map or discover or find spatial anchors. The 5G system shall enable an authorized third party to obtain all the spatial anchors in a given three dimensional area.

NOTE 1:	How an authorized third party identifies which three-dimensional area to request spatial anchors in is not in scope of the 3GPP standard. Spatial localization and mapping information could be used to identify areas of interest.

[R-5.2.1-004] Subject to operator policy and regulatory requirements, the 5G system shall support mechanisms to expose a spatial map or derived localization information to authorized third parties.

SA6 already supports SEAL layer. SA6 can enhance the existing SEAL capability (like location management) to enable support for spatial mapping.

### 4.4.2	Open issues

SA6 defined enablers as of now do not support operations required for spatial mapping. The open issues are:

1)	How to produce or modify a spatial map by the consumer (e.g. UE, VAL server)?

2)	How to expose a spatial map to authorized third parties?

## 4.5	Key issue #5: Support for avatar discovery and QoS control

### 4.5.1	Description

Mobile Metaverse brings a new dimension to XR/VR services (since meta is about a persistent large-scale virtual interactive experience, where the digital assets are owned or deployed by the end users); and requires enhancements to 3GPP systems to ensure connectivity /performance and provide support.

Two relevant use case as described in TS 22.156 [2] are about the 1) Mobile Metaverse Based Selective Multi-modal Feedback Service and 2) Mobile Metaverse for 5G-enabled Traffic Flow Simulation and Situational Awareness. For instance, for the latter use case, to support traffic flow simulation and situational awareness service, the 5G network need to provide low latency, high data rate and high reliability transmission, and in addition, the 5G network may also need to be further enhanced to meet the service requirements for 5G-enabled traffic flow simulation and situation awareness. Meanwhile, in addition to the real objects which may host the UE, their corresponding virtual objects are also capable of interacting with each other and interact with physical objects via 5GS.

For such scenarios, there are some issues related to how the digital devices are instantiated and onboarded to edge/cloud platforms (supporting mobile metaverse services) and how a digital device discovers other digital devices within the mobile metaverse service.

Furthermore, in mobile metaverse scenarios there can be multiple sessions involved. The figure below shows an example of possible sessions for a mobile metaverse service.

Figure 4.5.1-1: Example use case for multiple sessions in mobile metaverse service

There can be multiple multimodal sessions considering:

1) the interaction between the UE in the physical world and the digital UE at the metaverse (for providing sensor data/measurements and getting multimodal feedback) (green and blue links).

2) interaction between digital UEs (or avatar UE) interacting at the metaverse world. Such avatars can be hosted in the same or different edge/cloud platforms or at the ASP domain.

3) interaction between UE1 and UE2 via the network (which are in vicinity in metaverse but can be far away and served by different RATs/networks) for transactions between the UEs within the metaverse session.

4) interaction of metaverse server with the UE1 and UE2 avatars to configure the interactions for the metaverse service and provide the digital environment as well as to provide ASP policies for the interactions.

Such multitude of sessions (with different traffic requirements) within the same service poses some issues related to how QoS is being monitored and coordinated considering the end-to-end mobile metaverse service requirements.

### 4.5.2	Open issues

This Key Issue aims to study:

-	How to support the discovery and grouping of digital/avatar and physical UEs so as to interact via the mobile network?

-	How to monitor and configure the application QoS parameters for the multimodal sessions within meta service?

NOTE 1:	Coordination with FS_XRApp may be required for the QoS aspects impacting the multimodal sessions

## 4.6	Key issue #6: Support device discovery to offload task for metaverse services

### 4.6.1	Description

SA1 studied use cases to support metaverse services in 3GPP TR 22.856 [3]. However, many of the devices may not be equiped with sufficient capacity to process the data. And so, they need support from other devices to process data as much near as possible.

In order to provide immersive interactive location agnostic service experience to mobile metaverse service customers, large amount of computing resouces is needed to perform real-time processing for audio, video, and interactive data, etc. It is possible that some of the devices (e.g. glasses) will not have enough computing resources to perform the real-time rendering, and may need to offload the computing to the nearby devices.

For example, SA6 has defined architecture for Application layer support for Personal IoT Network in 3GPP TS 23.542 [12]. To support above use case, a PINE element (i.e. glasses with limited computing capability) need to discover another nearby PINE element (i.e. a UE with sufficient computing capability) to offload the computational task. Current PINAPP architecture supports subscribe-notify mechanism where a PINE element gets notification when a new PINE element joins the PIN. However, not all devices may subscribe for such event. It is required to study how a PINE element (i.e. glasses with limited computing capability) discovers another nearby PINE elements.

### 4.6.2	Open issues

In this key issue, SA6 will study following open issues:

1)	Whether and how the existing SA6 defined enabler (e.g. PINAPP) is sufficient to support metaverse applications or any enhancements are required (e.g. to discover device from anothter device to offload task)?

## 4.7	Key issue #7: Support for metaverse services requiring multiple devices

### 4.7.1	Description

SA1 studied use cases to support metaverse services in 3GPP TR 22.856 [3]. In many of the use case, metaverse applications are defined at edge to provide law latency, and further, a user needs multiple devices inorder to work with metaverse services.

For example, consider a scenario where a metaverse service (e.g. a game service) requires haptic device associated with the UE/user in order for user to consume the service (e.g. to play the game). Current discovery mechanism (e.g. EAS discovery mechanism) do not support such application server requirements to be considered in the discovery procedure. The possible gap is to consider required capabilities of supported devices (like having joystic or haptic device) to run specific metaverse service.

The existing SA6 defined enabler layer procedures are generic for all applications (e,g, EDGEAPP architecture).

It is required to study

-	How the interaction among multiple devices and AS can be supported in order to provide or to consume metaverse service;

### 4.7.2	Open issues

In this key issue, open issues are:

1)	To study whether and how to enhance architecture and procedures (e.g. EDGEAPP) to support metaverse service requiring multiple devices?

## 4.8	Key issue #8: Enabler support for spatial anchors based services

### 4.8.1	Description

SA1 in 3GPP TS 22.156 [2] defines spatial anchors as "an association between a location in space (three dimensions) and service information that can be used to identify and access services, e.g. information to access AR media content".

Application providers can use spatial anchors to associate application content and services with location information which can be managed by the 3GPP system. In turn, spatial anchors can be used by application clients to discover and access application content and services in a location aware fashion.

Clause 5.1 "Use Case on Localized mobile Metaverse serivce" of 3GPP TR 22.856 [3] provides a detailed use case for spatial anchor enabled services. It describes the use case where multiple spatial anchors and their services are linked to the space that the user is in. In the usecase, it is mentioned that one or more mobile metaverse services associated with a single location, to be combined to form a single location related service experience.

The clause 5.1 use case describes the example of recommending a spatial anchor of a restaurant to the user in the evening time in order to provide an enhanced service experience to the user based on the previous search history spatial anchor. This creates the need for the enabler to provide some intelligent spatial anchor service recommendations which could help the user and improve the service experience.

The consumers created spatial anchors may want to understand the analytics of the spatial anchors like users density per spatial anchor, spatial anchor density in a given location, number of times the spatial anchor is accessed to improve  their services, spatial anchor improvements like spatial anchor placement planning, user engagement, etc.

### 4.8.2	Open issues

The open issues are:

1)	How to combine one or more mobile metaverse services associated with spatial anchors?

2)	How to provide the exposure of the spatial anchor analytics to the consumers?

## 4.x	Key issue #x: <Title>

### 4.x.1	Description

Editor's Note:	This clause will describe the key issue.

### 4.x.2	Open issues

Editor's Note:	This clause will provide the list of open issue(s).

# 5	Application enablement architecture requirements

## 5.1	General requirements

Editor's Note:	This clause will describe general architectural requirements.

## 5.x	<application enabler layer capability x> requirements

Editor's Note:	This clause will describe the architectural requirements for the studied application layer capabilities.

# 6	Application enablement architecture for metaverse services

## 6.1	Option #1: On-network mobile metaverse application layer architecture

This clause proposes a mobile metaverse enablement layer as a possible architecture.

Editor's Note: Whether the mobile metaverse architecture will be an enablement layer, or a new SEAL service, or an enhancement to an existing SEAL service will be decided in the conclusion of this study.

### 6.1.1	Application enablement architecture

Figure 6.1.1-1 illustrates the detailed on-network mobile metaverse application layer functional model.

Figure 6.1.1-1: On-network mobile metaverse application layer functional model

The mobile metaverse application layer functional entities are grouped into the vertical application layer and the mobile metaverse application enablement layer. The mobile metaverse application enablement layer offers the mobile metaverse application enablement capabilities to the vertical application layer. The mobile metaverse application layer functional model utilizes SEAL services as specified in 3GPP TS 23.434 [8].

The mobile metaverse application enablement layer functional entities on the UE and in the network are respectively the mobile metaverse enabler client (MMEC) and mobile metaverse enabler server (MMES). The MMEC communicates with the MMES over the MM-UU reference point. The MMEC provides functionality to the VAL client(s) over the MM-C reference point. The VAL server(s) communicate with the MMES over the MM-S reference point. The MMES communicates with other MMES instances over the MM-E reference point. The MMES communicates with the underlying 3GPP network system using 3GPP interfaces specified by the 3GPP network system.

### 6.1.2	Functional Elements

#### 6.1.2.1	Mobile Metaverse Enablement Server

A Mobile Metaverse Enablement Server (MMES) provides metaverse application enablement services used by Metaverse VAL servers and Mobile Metaverse Enablement Clients (MMEC).

#### 6.1.2.2	Mobile Metaverse Enablement Client

A Mobile Metaverse Enablement Client provides metaverse application enablement services to VAL clients.

### 6.1.3	Reference Points

#### 6.1.3.1	MM-UU

MM-UU reference point enables interactions between a MMES and MMEC.

#### 6.1.3.2	MM-S

MM-S reference point enables interactions between a MMES and Metaverse VAL servers.

#### 6.1.3.3	MM-E

MM-E reference point enables interactions between MMES instances.

#### 6.1.3.4	MM-C

MM-C reference point enables interactions between a MMEC and Metaverse VAL clients.

## 6.2	Option #2: A-DACM architecture to support Metaverse services

### 6.2.1	Application enablement architecture

Figure 6.2.1-1 illustrates the A-DACM architecture to support Metaverse services.

Figure 6.2.1-1: A-DACM architecture to support Metaverse services

The SEAL architecture includes a A-DACM function which supports the VAL layer. The A-DACM function interacts with the VAL server(s) over the SEAL-S reference point. The A-DACM function may utilize the 5GC services over N33 reference point.

Editor's Note: How VAL client(s) access the services of A-DACM function is FFS.

Figure 6.2.1-2 illustrates the service-based SEAL architecture to support Metaverse services.

Figure 6.2.1-2: Service-based SEAL architecture to support Metaverse services

The A-DACM function exposes the APIs over the Sdacm interface to the VAL layer. The VAL functions (VAL servers) consume the services by interacting on the Sdacm interface of the A-DACM function. The CAPIF can be used by VAL functions to discover the services of the A-DACM function.

NOTE:	The A-DACM function can be realized as a new SEAL service or part of existing SEAL service.

### 6.2.2	Functional Elements

#### 6.2.2.1	A-DACM function

The A-DACM function expose the services/APIs to the VAL layer services over Sdacm interface for the following avatar management capabilities:

-	Avatar profile creation;

-	Avatar upload;

-	Avatar linking to user/subscriber;

-	Avatar download;

-	Avatar update/modification;

-	Avatar delete;

### 6.2.3	Reference Points

#### 6.2.3.1	SEAL-S

This reference point supports the interactions between VAL server(s) and A-DACM function. The VAL server consumes the avatar management services supported by the A-DACM function as specified in clause 6.x.2.1.

### 6.2.4	Service-based interface

#### 6.2.4.1	Sdacm

This interface supports the interactions between VAL function (VAL servers) and A-DACM function. The VAL function consumes the avatar management services supported by the A-DACM function as specified in clause 6.x.2.1.

## 6.x	Option #x: <title>

### 6.x.1	Application enablement architecture

Editor's Note:	This clause will illustrate the high level architecture and possible architecture enhancements for supporting metaverse services.

### 6.x.2	Functional Elements

Editor's Note:	The functional elements corresponding to the architecture will be presented in this clause.

### 6.x.3	Reference Points

Editor's Note:	The reference points corresponding to the architecture will be presented in this clause.

# 7	Solutions

## 7.0	Mapping of solutions to key issues

Table 7.0-1: Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #1 | X |  |  |  |
| Sol #2 | X |  |  |  |
| Sol #3 | X |  |  |  |
| Sol #5 |  |  | X |  |
| Sol #8 |  |  |  | X |

## 7.1	Solution #1: Spatial anchor discovery

### 7.1.1	Solution description

#### 7.1.1.1	General

This solution is for KI#1 and addresses the open issue of spatial anchor discovery by the consumer.

This solution is based on the Mobile Metaverse enablement layer architecture.

Editor's Note: Whether the mobile metaverse architecture will be an enablement layer, or a new SEAL service, or an enhancement to an existing SEAL service will be decided in the conclusion of this study, and this solution will be adapted to the chosen architecture.

In this solution, the mobile metaverse application enablement layer provides capabilities to discover and determine the validity of spatial anchors. Spatial anchors may be discovered and validated based on consumer location and orientation, consumer capabilities and preferences, and spatial anchor configuration and policy.

#### 7.1.1.2	Procedure

Pre-conditions:

1)	Spatial anchors have been provisioned in the mobile metaverse enabler server

2)	A VAL client (e.g., an AR application) needs to display enriched localized content to a user

Figure 7.1.1.2-1: Spatial anchor discovery procedure

0.	A metaverse VAL client discover s spatial anchors. The VAL client performs spatial anchor discovery using the mobile metaverse enabler client (MMEC) of the UE. The VAL client provides spatial anchor discovery requirements to the MMEC to discover spatial anchors of interest. The discovery requirements include a VAL client identifier, a VAL client type, information on location(s) of interest, and information on service(s) of interest (e.g., service type, service provider identifier, time of day when service is requested, etc.) for the service associated with spatial anchors, and indication to enable persistent search.

NOTE 1: The requirements for spatial anchor discovery may be further defined in the normative phase.

1.	The MMEC sends a spatial anchor discovery request to the mobile metaverse enabler server (MMES). The request includes information elements defined in Table 7.1.3-1.

2.	Upon receiving the request, the MMES validates if the requestor is authorized to discover spatial anchors. If the requestor is authorized, the MMES uses the information included the spatial anchor discovery request to determine spatial anchors to include in the discovery response.

If spatial anchor discovery filters are provided in the request, the MMES determines any spatial anchors matching the discovery filters. For each determined spatial anchor, the MMES checks whether the requestor is authorized to discover the spatial anchor based on the identity and location of the requestor and the access controls and service area characteristics of the spatial anchor. To determine a location of interest to the requestor, the MMES may use location information in the request, if available, or obtain location information by invoking the 3GPP Core Network Location Services exposed by the NEF as described in TS 23.273 [7] and TS 23.502 [9], or by invoking the SEAL Location Management APIs as described in TS 23.434 [8]. Additionally, the MMES may obtain and consider data analytics, such as UE mobility prediction, during spatial anchor determination. The MMES may obtain data analytics by invoking NEF APIs as described in TS 23.288 [10] and TS 23.502 [9].

3.	The MMES sends a spatial anchor discovery response to the MMEC. If the MMES has successfully determined spatial anchors, the response may include a success indication, a list of the determined spatial anchors as defined in Table 7.1.3-2. If the MMES has not determined any spatial anchor, the response may include a failure indication and a cause of failure. The response also includes the recommended list of spatial anchors in the discovery response based on the discovery filter information and stored spatial anchor information.

NOTE 2:	The logic to generate the recommended list of spatial anchor is out of scope and implementation-specific.

4.	Upon receiving the discovery response, if the response indicates success, the MMEC may store the received spatial anchor information in a spatial anchor cache. The MMEC evaluates the validity conditions of discovered spatial anchors.

5.	The MMEC provides the valid spatial anchor information to the VAL client(s) based on VAL client requirement(s). Upon receiving valid spatial anchor information, the VAL client can access the service(s) associated with each spatial anchor.

NOTE 3:	This procedure may be adapted to allow a VAL server to discover spatial anchor(s).

### 7.1.2	Architecture Impacts

Option #1 in clause 6.1 is the basis for this solution.

### 7.1.3	Corresponding APIs

Table 7.1.3-1 shows the request sent by a MMEC to a MMES for the discovery of spatial anchor(s).

Table 7.1.3-1: Spatial anchor discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., MMEC, UE, VAL server) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Location of interest | O | Location of spatial anchors the requestor is interest in. |
| Spatial anchor discovery filters | O | Set of characteristics to determine matching spatial anchors (e.g., spatial anchor location, VAL client type). |

Table 7.1.3-2 shows the response sent by the MetaApp server to the requester for a spatial anchor discovery request.

Table 7.1.3-2: Spatial anchor discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request (e.g., success or fail), including failure reason if needed. |
| Discovered spatial anchor List | O | List of discovered spatial anchor(s). Each element includes the information described below |
| > Spatial anchor ID | M | Identifier of the spatial anchor. |
| > Spatial anchor location | M | Location of the spatial anchor (e.g., GPS coordinates, civic address, etc.). |
| > Spatial anchor validity criteria | M | The validity criteria of a spatial anchor (e.g., spatial criteria, time criteria, etc.). |
| > VAL server endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) of the service(s) associated with the spatial anchor. |
| > VAL server information | O | Information about the service offered by the VAL server (e.g., service description, service name, etc.). |

### 7.1.4	Solution evaluation

Editor's Note:	Evaluation of this solution is FFS.

## 7.2	Solution #2: Support for spatial anchor management

### 7.2.1	Solution description

This solution maps to KI# 1. This solution enhances Service Enabler Architecture Layer (SEAL) Location Management (LM) service to provide spatial anchor management service to the VAL server or the UE. The architecture impacts are captured in clause 7.2.2 while procedures to manage spatial anchors are captured in clause 7.2.3.

### 7.2.2	Architecture Impacts

This clause provided architecture enhancements to the SEAL LM architecture as defined in 3GPP TS 23.434 [8]. The SEAL LM client is enhanced to provide client side functionalities for managing (i.e. create, read, update and delete) spatial anchors for metaverse application. The SEAL LM server is enhanced to provide server side functionalities for managing spatial anchors for the metaverse applications.

VAL client uses the SEAL LM client to manage spatial anchors for the metaverse applications over LM-C reference point. Similarly, VAL server uses SEAL LM server to manage spatial anchors for the metaverse applications over LM-S reference point. The LM-UU interface is enhanced to provide management of spatial anchors between SEAL LM client and SEAL LM server.

Editor's Note: Whether to use a new application enablement server or a new SEAL server for this solution is FFS.

### 7.2.3	Procedures

#### 7.2.3.1	Creating spatial anchor

Figure 7.2.3.1-1 depicts the procedure for creating association of spatial anchor with location for metaverse applications. The service is provided by SEAL LM server and consumed by VAL server or SEAL LM client.

Figure 7.2.3.1-1: Create spatial anchor

1)	The VAL server (or SEAL LM client) sends a request message to the SEAL LM server to create association of the spatial anchor with the location. The request includes VAL server identity, position of the anchor and application service identifier, security credentials, spatial anchor service area. The request may include service information of the product to associate it with the spatial anchor, access control rules defining which entities are permitted to discover and access the spatial anchor, and customer premise information (e.g. a residence, office or shop).

2)	The SEAL LM server authorizes VAL server (or SEAL LM client). If the requestor is authorized, then the SEAL LM server creates the association between spatial anchor and the location, and stores the information as received in the request message. The server creates identity for the spatial anchor. The SEAL LM server sends the response back to the VAL server (or SEAL LM client) including identity for the spatial anchor for a successful case.

#### 7.2.3.2	Updating spatial anchor

Figure 7.2.3.2-1 depicts the procedure for updating spatial anchor association for metaverse applications. The update operation includes modifying existing spatial anchor or deleting the spatial anchor. The service is provided by SEAL LM server and consumed by VAL server or SEAL LM client.

Figure 7.2.3.2-1: Update spatial anchor

1)	The VAL server (or SEAL LM client) sends a request message to the SEAL LM server to update the existing spatial anchor association. The request is used to modify or delete the spatial anchor. The request message includes the identity of the spatial anchor and the application service identifier. To modify the spatial anchor, the request may include service information of the product to associate it with the spatial anchor, spatial anchor service area and access control rules, and customer premise information (e.g. a residence, office or shop).

2)	The SEAL LM server authorizes VAL server (or SEAL LM client). If the requestor is authorized, then the SEAL LM server checks whether the spatial anchor as identified by the identity of the spatial anchor exists or not. If the spatial anchor does not exists then the SEAL LM server sends failure response. Otherwise, the SEAL LM server updates the spatial anchor details (for update request) or deletes the spatial anchor details (for delete request). The SEAL LM server sends the response back to the VAL server (or SEAL LM client) including reason for failure for failure case.

#### 7.2.3.3	Get spatial anchor

Figure 7.2.3.3-1 depicts the procedure for getting spatial anchor details for metaverse applications. The service is provided by SEAL LM server and consumed by VAL server or SEAL LM client.

Figure 7.2.3.3-1: Get spatial anchor

1)	The VAL server (or SEAL LM client) sends a request message to the SEAL LM server to get the spatial anchor details. The request includes discovery filters like the position of the user and application service identifier. If the request to get specific spatial anchor, the request may include application specific information (e.g type of service).

2)	The SEAL LM server authorizes VAL server (or SEAL LM client). If the requestor is authorized, then the SEAL LM server determines all spatial anchors from the repository based on the discovery filters provided in the request message. The SEAL LM server sends the response back to the VAL server (or SEAL LM client) including list of spatial anchor details for successful case.

#### 7.2.3.4	Spatial anchor information subscription

Figure 7.2.3.3-1 depicts the procedure for subscribing to receive spatial anchor details for metaverse applications. The service is provided by SEAL LM server and consumed by VAL server or SEAL LM client.

Figure 7.2.3.4-1: Spatial anchor information subscription

1)	The VAL server (or SEAL LM client) sends a subscription request to the SEAL LM server to get notifications about the spatial anchor details. The request includes discovery filters like the position and application service identifier. If the request to subscribe to specific spatial anchor, the request may include application specific information (e.g. type of service).

2)	The SEAL LM server authorizes VAL server (or SEAL LM client). If the requestor is authorized, then the SEAL LM server sends successful response along with subscription identity.

NOTE 1:	The SEAL LM server starts monitoring the location of the user if it is not already monitoring.

3-4)	Upon occurance of the event (e.g. spatial anchor added or removed or updated), the SEAL LM server determines all spatial anchors from the repository which are matched with the discovery filters provided in the request. The SEAL LM server sends the notification to the VAL server (or SEAL LM client) including list of spatial anchor details. The response may include the recommended list of spatial anchor details based on the requested spatial anchor discovery filter, application specific information (e.g. type of service information) and the stored spatial anchor(s).

NOTE: The logic to provide a recommended list of spatial anchor details is implementation-specific and out of scope

### 7.2.4	Solution evaluation

Editor's Note:	This clause provides an evaluation of the solution addressing KI#x.

## 7.3	Solution #3: Spatial anchor subscription

### 7.3.1	Solution description

#### 7.3.1.1	General

This solution is for KI#1 and addresses the open issue of how to detect UE(s) within a certain range of a spatial anchor and detect spatial anchors within a certain range of a UE.

This solution is based on the Mobile Metaverse enablement layer architecture.

Editor's Note: Whether the mobile metaverse architecture will be an enablement layer, or a new SEAL service, or an enhancement to an existing SEAL service will be decided in the conclusion of this study, and this solution will be adapted to the chosen architecture.

A first procedure is proposed to allow a VAL server to subscribe for information about UE with spatial anchor interest. The mobile metaverse application enablement layer provides capabilities to subscribe for notifications providing information on UE with spatial anchor interest and that may enter, exit or be in range of spatial anchor of interest. The  notification may include information about UEs that have discovered spatial anchor(s) but may not yet have connected to the service associated with the spatial anchor(s).

A second procedure is proposed to allow a UE (e.g., MMEC, VAL client) to subscribe for information about spatial anchors of interest. The mobile metaverse application enablement layer provides capabilities to subscribe for notifications providing information on spatial anchor interest and that may be in range, become in range or become out of range of the UE.

#### 7.3.1.2	VAL server subscription procedure for information about UE with spatial anchor interest

This procedure goes beyond capabilities defined in SEAL LMS for providing UEs within range of a target location described in TS 23.434 [8]. SEAL LMS provides information about UE(s) within range of a location, which may be the location of a spatial anchor, regardless of the interest of such UE(s) for spatial anchors. This solution adds spatial anchor awareness such that UE(s) that consume spatial anchors and have discovered spatial anchors of interest are considered. Spatial anchor awareness allows a VAL server (e.g., that may have provisioned spatial anchors or may offer a service associated with a spatial anchor) to discover UE(s) with interest in the associated spatial anchors. SEAL LMS and this solution can be used in conjunction by a VAL server.

Pre-conditions:

1)	Spatial anchors have been provisioned in the mobile metaverse enabler server

2)	A VAL server requires information about UEs with spatial anchor interest.

Figure 7.3.1.2-1: UE with spatial anchor interest subscription/notification procedure

1.	A metaverse VAL server sends a UE with spatial anchor interest subscription request to the mobile metaverse enabler server (MMES). The request includes the VAL server identifier, security credentials, a VAL server notification endpoint (e.g., IP address, FQDN, URI), and spatial anchor of interest detection filters. The detection filters may include spatial anchor identifiers to detect UEs that have discovered specific spatial anchors, spatial anchor associated service identifiers to detect UEs that have discovered spatial anchors associated with a service identifier, location area to detect UEs that have discovered spatial anchors within a location area, spatial anchor range to detect UEs that have discovered and are within range or become out of range of spatial anchors. The subscription may also indicate whether the notification is for a UE entering the range, leaving the range or both.

2.	Upon receiving the request, the MMES authorizes the VAL server subscription request and performs the necessary actions to detect UE with spatial anchor interest. To detect UEs entering or leaving the range of spatial anchor of interest, the MMES considers if the spatial anchors of interest have been discovered by a UE and obtains the corresponding UE location information. The UE location information may be obtained by using capabilities of the 5GS by invoking the 3GPP Core Network Location Services exposed by the NEF as described in TS 23.273 [7] and TS 23.502 [9], or by invoking the SEAL Location Management APIs as described in TS 23.434 [8]. Additionally, the MMES may consider data analytics, such as UE mobility prediction. The MMES may obtain data analytics by invoking NEF APIs as described in TS 23.288 [10] and TS 23.502 [9]. The MMES may monitor the distance between UEs and the spatial anchor of interest to determine whether the UEs enters, exits or is within range of the spatial anchors of interest.

3.	The MMES sends a UE with spatial anchor interest subscription response to the VAL server. The response includes an indication of success or failure for the subscription. If successful, the response includes a subscription identifier; otherwise, the response includes an error code and reason of failure.

4.	Based on the detection filters provided in the subscription request, the MMES detects UE(s) with interest in spatial anchors (e.g., UE that discovers the monitored spatial anchor), and identifies whether the UE enters or exits the monitored spatial anchor range.

5.	The MMES sends a UE with spatial anchor interest notification to the subscribed VAL server; the notification includes the subscription identifier and information about the detection of UE with spatial anchor interest (e.g., UE identifier, spatial anchor identifier(s), range from the spatial anchor(s), entering or exiting the range, etc.).

#### 7.3.1.3	VAL client subscription procedure for spatial anchors in range

Pre-conditions:

1)	Spatial anchors have been provisioned in the mobile metaverse enabler server

2)	A VAL client requires information about spatial anchors of interest

Figure 7.3.1.3-1: Spatial anchor of interest subscription/notification procedure

1.	A metaverse VAL client subscribes for spatial anchor of interest information with the mobile metaverse enabler client (MMEC), and the subscription includes a VAL client identifier and spatial anchor requirements. The spatial anchor requirements includes spatial anchor of interest detection filters and a range. The detection filters may include spatial anchor identifiers to detect specific spatial anchors, spatial anchor associated service identifiers to detect spatial anchors associated with a service identifier, location area to detect spatial anchors within a location area, range to detect spatial anchors within a range, an indication to enable persistent search to notify spatial anchors within range of UE’s location. The subscription may also indicate whether notifications are for spatial anchors becoming in range of the UE, becoming out of range of the UE or both.

2.	The MMEC sends a spatial anchor of interest subscription request to the mobile metaverse enabler server (MMES). The request includes the MMEC identifier, VAL client identifier, security credentials, a notification endpoint (e.g., IP address, FQDN, URI), and spatial anchor of interest detection filters provided by the VAL client.

3.	Upon receiving the request, the MMES authorizes the MMEC subscription request and performs the necessary actions to detect spatial anchor of interest. To detect spatial anchor of interest within range or becoming in-range or out-of-range of the UE, the MMES obtains the corresponding UE location information. The UE location information may be obtained by using capabilities of the 5GS by invoking the 3GPP Core Network Location Services exposed by the NEF as described in TS 23.273 [7] and TS 23.502 [9], or by invoking the SEAL Location Management APIs as described in TS 23.434 [8]. Additionally, the MMES may consider data analytics, such as UE mobility prediction. The MMES may obtain data analytics by invoking NEF APIs as described in TS 23.288 [10] and TS 23.502 [9]. The MMES may monitor the distance between UEs and the spatial anchor of interest to determine whether the UEs becomes in-range or out-of-range of the spatial anchors of interest.

4.	The MMES sends a spatial anchor of interest subscription response to the MMEC. The response includes an indication of success or failure for the subscription. If successful, the response includes a subscription identifier; otherwise, the response includes an error code and reason of failure.

5.	Based on the detection filters provided in the subscription request, the MMES monitors the distance between the UE and the spatial anchor of interest to determine whether the spatial anchor of interest becomes in-range or out-of-range of the UE.

6.	The MMES sends a spatial anchor of interest notification to the subscribed MMEC; the notification includes the subscription identifier and information about the detection of spatial anchors of interest (e.g., spatial anchor identifier(s), range from the UE, in-range, out-of-range, etc.).

7-	The MMEC provides to the VAL client the information received in the notification about the spatial anchors of interest.

### 7.3.2	Architecture Impacts

Option #1 in clause 6.1 is the basis for this solution.

### 7.3.3	Corresponding APIs

Editor's Note:	APIs of this solution are FFS.

### 7.3.4	Solution evaluation

Editor's Note:	Evaluation of this solution is FFS.

## 7.4	Solution #4: Support for spatial anchor analytics information

### 7.4.1	Solution description

This solution maps to KI# 1.

### 7.4.2	Procedures

#### 7.4.2.1	Spatial anchor analytics information (subscription-notify)

Figure 7.4.2.1-1 depicts the procedure for subscribing to receive spatial anchor analytics information for metaverse applications. The service is provided by MMES server and consumed by VAL server.

Figure 7.4.2.1-1: Spatial anchor analytics information subscription

1.	A metaverse VAL server sends a spatial anchor analytics subscription request to the mobile metaverse enabler server (MMES). The request can include the VAL server identifier, security credentials, a VAL server notification endpoint (e.g., IP address, FQDN, URI), and spatial anchor analytics filters like spatial anchor identifers, spatial anchor location information, position of the user, proximity distance, and application service identifier. It may also include the periodicity for reporting notification and analytics sampling interval.

2.	Upon receiving the request, the MMES authorizes the VAL server subscription request and subscribes the 5GS  NEF services, NWDAF, ADAE services for collecting analytics as per the analytics filter. The UE location information may be obtained by using capabilities of the 5GS by invoking the 3GPP Core Network Location Services exposed by the NEF as described in TS 23.273 [7] and TS 23.502 [9], or by invoking the SEAL Location Management APIs as described in TS 23.434 [8]. Additionally, the MMES may consider data analytics, such as UE mobility prediction, application session analytics. The MMES may obtain data analytics by invoking NEF APIs as described in TS 23.288 [10] and TS 23.502 [9].

The MMES sends a spatial anchor analytics subscription response to the VAL server. The response includes an indication of success or failure for the subscription. If successful, the response includes a subscription identifier; otherwise, the response includes an error code and reason of failure.

3.	The MMES generates the spatial anchor-related analytics based on the obtained data and the stored spatial anchor data as per the analytics sampling interval. It provides response to the VAL server as per the reporting interval. The response message includes spatial anchor ID, number of times spatial anchor discovered and accessed, spatial anchor density per location, user density per spatial anchor, spatial anchor services accessed information, spatial anchor services session time per user.

## Editor's Note: Whether Metaverse enabler layer can perform spatial anchor analytics or ADAES needs to be enhanced for supporting step 3 is FFS.

## 7.5	Solution #5: Support for digital avatars

### 7.5.1	Solution description

Metaverse is considered as a digital world which is a replica of a real world. Most of the Metaverse application needs avatar for the user to interact with the application. And for each application, user may required to create, modify, get and delete avatars. For each avatar (regardless of application) will have some common properties. Depending on the metaverse application from where the user is interested to take the service, he/she can choose his/her avatar and the related information when needed. Also, a user can move between metaverse applications using the same avatar seamlessly and taking into account the constraints of the visited application.

This solution maps to KI# 3. This solution provides support to manage digital avatars for the users.

### 7.5.2	Architecture Impacts

This clause provided architecture enhancements to the SEAL CM architecture as defined in 3GPP TS 23.434 [8]. The SEAL CM client is enhanced to provide client side functionalities for managing (i.e. create, read, update and delete) application specific avatar profile(s) for metaverse application. The application specific avatar profile(s) contains avatar related properties and configurations.

The SEAL CM server is enhanced to provide server side functionalities for managing application specific avatar profile(s) for the metaverse applications. It allows the consumer of the service to create, update, fetch, and delete the application specific avatar profile(s). The application specific avatar profile(s) can be stored into VAL user database.

VAL client uses the SEAL CM client to manage application specific avatar profile(s) for the metaverse applications over CM-C reference point. Similarly, VAL server uses SEAL CM server to manage application specific avatar profile(s) for the metaverse applications over CM-S reference point. The CM-UU interface is enhanced to provide management of application specific avatar profile(s) between SEAL CM client and SEAL CM server.

A user can have one or more application specific avatar profile(s). An application specific avatar profile can be shared by one or more users (for example, an avatar profile is created for a sales representative, and used by all sales personals).

Editor's Note: Whether to use a new application enablement server or a new SEAL server for this solution is FFS.

### 7.5.3	Procedures

#### 7.5.3.1	Create application specific avatar profile

The procedure for creating avatar profile is illustrated in figure 7.5.3.1-1. The configuration management server exposes the API for the consumer to create the avatar profile(s). The application specific avatar profile(s) may be created based on user’s request via configuration management client or based on request from the VAL server.

The application specific avatar profile(s) may be create for a single user or it can be used by multiple users. For application specific avatar profile(s) which can be used by multiple users, the users can use the avatar at a same time simultaneously or one at a time depending on the application requirement. The configuration management server stores the avatar and its related properties and configuration.

Pre-conditions:

-	The VAL UE has the secure access to the configuration management server.

Figure 7.5.3.1-1: Creating application specific avatar profile

1.	The configuration management client sends a create avatar profile request to the configuration management server for creating avatar profile. The request includes required security credentials and other parameters to create avatar profile. The security credentials includes security token. The request may include allowed user list, current status (in use or not), current position, allowed application list, allowed locations, associated digital wallets, associated digital accessories, one or more predictive model, creation time, expiry time till when the avatar profile is valid, history information like at which locations avatar is used, which applications used the avatar profile, time of the use, etc.

Table 7.5.3.1-1: Create avatar profile request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor Id | M | Identify of the requestor (VAL UE or VAL server). |
| Security parameters | M | Security parameters for authentication and authorization. |
| Application specific avatar profile | M | Application specific avatar profile |
| > user ID | M | Identity of the user for which the profile is created |
| > allowed user list | O | List of users allowed to use this avatar |
| > current status | O | Current status (in use or not) of the profile |
| > allowed locations | O | List of locations where this avatar profile is allowed to use |
| >> current location | O | Current location where the profile is being used |
| > allowed application list | O | List of application IDs which are allowed to use this avatar profile |
| > associated accessories | O | List of accessories (e.g. Hat, watch, shoes) which are purchased for this avatar profile |
| > list of predictive models (NOTE 1) | O | List of one or more predictive model for the user, in order to predict user’s behaviour. |
| > expiry time | O | Time till when this avatar profile is valid |
| > History information | O | Usage history for this avatar profile |
| >> list of locations | O | List of locations where the avatar profile is used. |
| >> time of use | O | Time information when the avatar profile is used |
| >> list of applications | O | List of the application IDs which used this avatar profile. |
| NOTE 1: This IE is only applicable if the avatar profile is not shared. |  |  |

Editor's Note: Associating the application specific avatar profile with SA2 defined avatar information is FFS.

Editor's Note: It is FFS whether the user ID IE is to be included as part of the Avatar Profile or is a separate digital asset associated with the avatar.

2.	The configuration management server authenticate and authorizes the user. If user is allowed to perform the operation then the configuration management server creates the resource to store the avatar profile. The configuration management server stores the profile as received in the request.

3.	The configuration management server sends create avatar profile response to the configuration management client. In case of success, the response also includes the identity of the avatar profile.

#### 7.5.3.2	Update application specific avatar profile

The procedure for updating avatar profile is illustrated in figure 7.5.3.2-1. The configuration management server exposes the API for the consumer to update the avatar profile. The application specific avatar profile(s) may be updated based on user’s request via configuration management client or based on request from the VAL server. This procedure is also used to link more users to the avatar profile.

Pre-conditions:

-	The VAL UE has the secure access to the configuration management server.

Figure 7.5.3.1-1: Creating application specific avatar profile

1.	The configuration management client sends a update avatar profile request to the configuration management server for creating avatar profile. The request includes required security credentials, identity of the avatar profile and parameters to update avatar profile. The security credentials includes security token. The other parameters includes allowed user list, current status (in use or not), current position, allowed application list, allowed locations, associated digital wallets, associated digital accessories, one or more predictive model, creation time, expiry time till when the avatar profile is valid, historical information like at which locations avatar is used, which applications used the avatar profile, time of the use, etc.

2.	The configuration management server authenticate and authorizes the user. If user is allowed to perform the operation then the configuration management server checks whether the avatar profile is present or not based on provided identity. If avatar profile is present, then the configuration management server updates the profile with the new parameters as provided in the request message.

3.	The configuration management server sends update avatar profile response to the configuration management client indicating success or failure of the procedure. In case of failure, the response includes the failure cause indicating that the profile does not exist.

#### 7.5.3.3	Get application specific avatar profile

The procedure to get avatar profile is illustrated in figure 7.5.3.3-1. The configuration management server exposes the API for the consumer to get the avatar profile.

Pre-conditions:

-	The VAL UE has the secure access to the configuration management server.

Figure 7.5.3.3-1: Get application specific avatar profile

1.	The configuration management client sends a get avatar profile request to the configuration management server for creating avatar profile. The request includes required security credentials and identity of the avatar profile.

2.	The configuration management server authenticate and authorizes the user. If user is allowed to perform the operation then the configuration management server checks whether the avatar profile is present or not based on provided identity.

3.	The configuration management server sends get avatar profile response to the configuration management client indicating success or failure of the procedure. In case of success, the response includes avatar profile with all required parameters. In case of failure, the response includes the failure cause indicating that the profile does not exist.

#### 7.5.3.4	Delete application specific avatar profile

The procedure to delete avatar profile is illustrated in figure 7.5.3.4-1. The configuration management server exposes the API for the consumer to delete the avatar profile.

Pre-conditions:

-	The VAL UE has the secure access to the configuration management server.

Figure 7.5.3.4-1: Delete application specific avatar profile

1.	The configuration management client sends a delete avatar profile request to the configuration management server for creating avatar profile. The request includes required security credentials and identity of the avatar profile.

2.	The configuration management server authenticate and authorizes the user. If user is allowed to perform the operation then the configuration management server checks whether the avatar profile is present or not based on provided identity. If the avatar profile is present then the configuration management server deletes the avatar profile.

3.	The configuration management server sends delete avatar profile response to the configuration management client indicating success or failure of the procedure. In case of failure, the response includes the failure cause indicating that the profile does not exist.

#### 7.5.3.5	Application specific avatar profile(s) subscription

Figure 7.5.3.5-1 depicts the procedure for application specific avatar profile(s) subscription. The service is provided by SEAL CM server and consumed by VAL server or SEAL CM client.

Figure 7.5.3.1-1: Application specific avatar profile(s) subscription

1)	The VAL server (or SEAL CM client) sends a subscription request to the SEAL CM server to get notifications about the application specific avatar profile(s). The request includes avatar ID, user ID, filter criteria like the application specific information (e.g., VAL user location, VAL client type, type of service), event (e.g. application specific avatar profile created or removed or updated).

2)	The SEAL CM server authorizes VAL server (or SEAL CM client). If the requestor is authorized, then the SEAL CM server sends successful response along with subscription identity.

3-4)	Upon occurrence of the event (e.g. application specific avatar profile created or removed or updated), the SEAL CM server determines all application specific avatar profile(s) from the repository which are matched with the filter criteria provided in the request. The SEAL CM server sends the notification to the VAL server (or SEAL CM client) including list of application specific avatar profile.

### 7.5.4	Solution evaluation

Editor's Note:	This clause provides an evaluation of the solution addressing KI#x.

## 7.6	Solution #6: Avatar support using Metaverse Digital Representation Block (MDRB)

### 7.6.1	Solution description

The following clauses specify procedures and information flows for Key Issue #3 for support of avatars, as a type of digital asset manged using Metaverse Digital Representation Block (MDRB).

MDRB is an information block containing one or more related digital assets, information about the associated users and about the associated metaverse services. MDRB also includes MMEL policies and parameters for the handling of the digital representation.

Editor's Note: Terminology alignment may be necessary.

Preconditions:

0.	Digital representations are available at the VAL application layer.

Figure 7.6.1.1: MDRB support request procedure

Procedural steps:

0.	The VAL server sends a request to the corresponding MMES to enable MDRB support for the metaverse application functionality related to the digital representation created. The message(s) include information about the digital representation(s) to be supported, e.g. avatar.

NOTE 1:	In the normative phase, the step 1 request can map to one or more requests, e.g. create /update /delete.

1.	The MMES determines applicable MMEL parameters for the handling of the digital representation. The MMEL support information and parameters are stored as MDRB.

NOTE 2:	The MDRB stores information necessary for access and manage the MDRB and its exposure to/from the VAL application layer. This information is provided by the VAL layer or is derived based on the VAL layer request and policies.

2.	Based on the information received from the VAL layer and on local policies, the MMES optionally determines to send one or more requests to the underlying 3GPP Network to determine information available in the 3GPP network for the MDRB.

NOTE 3:	The information available in the 3GPP network to be included as part of MDRB is to be determined in the normative phase and aligned with specifications from SA2, SA3 and SA4.

NOTE 4:	Step 3 may be triggered at any time after step 1.

3.	The MMES generates response and sends the message to the VAL Server.

### 7.6.2	Architecture Impacts

Editor's Note:	This clause provides the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 7.6.3	Corresponding APIs

Table 7.6.3-1 shows the MDRB support request from VAL Server to MMES.

Table 7.6.3-1: Request MDRB support

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Security credentials | M | Security credentials to authenticate and authorize the requestor. |
| MDRB identifier | O | An identifier for the MDRB for which MMEL support is requested |
| List of Metaverse service provider identifiers | M | List of service providers for the Metaverse services |
| List of associated user information (NOTE 4) | M | List of user information as detailed below. |
| > 3GPP subscriber association information (NOTE 1, NOTE 3) | O | This information is used in MMEL for interactions with 5GS which require subscriber/and/or user information. |
| > Metaverse service user information (NOTE 3) | O | Application-level information related to one or more application users. The information may be anonymized, etc. to meet the metaverse application security requirements. This information may represent or be provided by a third party. |
| List of digital assets (NOTE 4) | M | List of digital asset parameters for MDRB management. |
| > Asset type | M | Choice of asset type, e.g. avatar, wallet, etc. |
| > Asset profile information | O | Information used to obtain 5GS services, and which depends on asset type, metaverse use policies. For example, for an avatar asset, the information includes identifiers, etc necessary to utilize SA4 services |
| > Communication parameters | O | Communication parameters and requirements for sessions associated with the digital asset. The parameters are also used to create or derive the communication policies applicable to the asset which are persistent within metaverse |
| > Metaverse experience requirements | O | Information provided to create or derive associated metaverse experience parameters for each asset, e.g. information used to derive applicable rendering parameters |
| MRDB exposure application rules | O | Rules for exposure of MRDB information. The policy may include location, temporal, etc. conditions. |
| NOTE 1: IE to be determined in the normative phase based on SA2 work for user identity exposure NOTE 2: IE to be determined in the normative phase based on SA4 work for avatar services NOTE 3: IE to be determined in the normative phase including SA3 security considerations NOTE 4: Information exposed is based on applicable security requirements and policies in both application and network domains. |  |  |

Editor's Note: It is FFS how the MDRB identifier is assigned and used, beyond the role of identifier of the request.

Table 7.6.3-2 shows the MDRB support response from MMES to VAL Server.

Table 7.6.3-2: MDRB support response

| Information element | Status | Description |
| --- | --- | --- |
| MDRB identifier | M | Identifier of the MDRB for which support is provided |
| List of capabilities provided | M | List of MMEL capabilities available for support of the MDRB |
| Shareable MDRB information | O | MDRB information, which is shareable with the application domain. |
| MRDB common exposure policy` | O | Policy for exposure of MRDB information, determined based on the rules in MMEL service request, local or underlying network policies, etc. Individual exposure rules and policies can be additionally applied by 5GS. The policy may include location, temporal, etc. conditions. |
| NOTE: The shareable MDRB information provides applications with access to Core Network functionality currently under development, as well as to Service Layer metadata enabling the identification and coordination of these services. |  |  |

### 7.6.4	Solution evaluation

This solution addresses KI# 3 by providing a generic framework for Avatar support as a type of digital asset.

Editor's Note:	It is FFS whether generic support for digital assets (MDRB-based solution) is necessary to enable Avatar support.

## 7.7	Solution #7: Digital avatar support by A-DACM

### 7.7.1	Solution description

#### 7.7.1.1	General

This solution maps to KI#3. This solution is based on A-DACM function to support the following avatar management services offered to VAL layer (VAL server):

-	Avatar profile creation;

-	Avatar upload;

-	Avatar linking to user/subscriber;

-	Avatar download;

-	Avatar update/modification;

-	Avatar delete;

Editor's Note: How VAL client(s) can consume these services is FFS.

#### 7.7.1.2	Avatar profile creation

Figure 7.7.1.2-1 illustrates the high level flow for avatar profile creation procedure.

Figure 7.7.1.2-1: Procedure for Avatar profile creation

1.	VAL server sends the Create Avatar Profile Request to A-DACM function with the details such as Avatar ID (optional), Avatar properties (e.g. name), Avatar usage policies (e.g. how to use the avatars).

2.	The A-DACM function checks the authorization of the VAL server to perform the operation. If successful, then A-DACM function stores the avatar profile.

3.	The A-DACM function sends a Create Avatar Profile Response to the VAL server indicating success or failure of the operation.

#### 7.7.1.3	Avatar profile upload

Figure 7.7.1.3-1 illustrates the high level flow for avatar upload procedure.

Pre-conditions:

1.	Avatar ID is available as per the procedure in clause 7.7.1.2

Figure 7.7.1.3-1: Procedure for Avatar upload

1.	The VAL server sends the Upload Avatar Request to the A-DACM function with the Avatar ID and Avatar object/media information.

2.	The A-DACM function checks the authorization of the VAL server to perform the operation. If successful, the A-DACM function performs the storage of avatar object/media information else the storage of the avatar object/media information.

3.	The A-DACM function sends a Upload Avatar Response to VAL server indicating success or failure of the operation.

#### 7.7.1.4	Avatar linking to user/subscriber

Figure 7.7.1.4-1 illustrates the high level flow for avatar linking to user/subscriber procedure.

Figure 7.7.1.4-1: Procedure for Avatar linking to user/subscriber

1.	The VAL server sends a Link Avatar Request to the A-DACM function with Avatar ID and GPSI/External identifier of the UE.

2.	The A-DACM function checks the authorization of the VAL server to perform the operation. If successful, the A-DACM function checks for the authorization (e.g. user consent) of the subscriber (as per step 1) for linking the avatar with the user/subscriber. If authorized, the A-DACM function links the Avatar ID with the subscriber/user information and stores the related information.

3.	The A-DACM function sends a Link Avatar Response to the VAL server indicating success or failure of the operation.

#### 7.7.1.5	Avatar download

Figure 7.x.1.5-1 illustrates the high level flow for avatar download procedure.

Figure 7.7.1.5-1: Procedure for Avatar download

1.	The VAL server sends a Download Avatar Request to the A-DACM function with the Avatar ID, GPSI/External ID of the UE, Filter/Adaptation information (e.g.VAL information, media adaptations).

2.	The A-DACM function checks the authorization of the VAL server to perform the operation. If successful, the A-DACM function performs the media adaptation as per the additional filters/adaptation information in the request on the avatar object/media. The adapted avatar media/object is used in the further response to the VAL server.

3.	The A-DACM function sends a response to the VAL server indicating success or failure of the operation. If success, the avatar object/media is included in the response.

### 7.7.2	Architecture Impacts

The solution utilizes the architecture specified in clause 6.2, Option #2.

### 7.7.3	Corresponding APIs

Editor's Note:	This clause provides the corresponding APIs for supporting the solution.

### 7.7.4	Solution evaluation

This clause provides an evaluation of the solution addressing KI#3. The A-DACM function can support the avatar management services towards the VAL layer.

NOTE:	The security mechanisms for this solution is the responsibility of SA3.

## 7.8	Solution #8: Support for spatial map management

### 7.8.1	Solution description

This solution maps to KI#4. This solution enhances SEAL LM service to provide spatial map management service.

### 7.8.2	Architecture Impacts

This clause provides the architecture impacts of the solution. The solution provides enhancements to the SEAL LM functionalities for managing (i.e. produce, update, get, subscribe) spatial maps. To manage spatial maps between VAL server/SEAL LM client and SEAL LM server, reference points LM-S and LM-UU are used in this solution.

Editor's Note: Whether to use a new application enablement server such as a Mobile Metaverse Enabler Server (MMES) or a new SEAL server for this solution is FFS.

### 7.8.3	Procedures

#### 7.8.3.1	Producing spatial map

Figure 7.8.3.1-1 depicts the procedure for producing a spatial map. For the request from the spatial map requestor (VAL server or SEAL LM client), the service is provided by SEAL LM server.

Figure 7.8.3.1-1: Producing spatial map

1)	The VAL server (or SEAL LM client) sends a request message to the SEAL LM server to produce the spatial map. The request includes requestor ID, three-dimensional area of interest, information to be included in the spatial map.

Editor's Note: What information needs to be included in the spatial map is FFS.

The SEAL LM server authorizes VAL server (or SEAL LM client), and validates the request.

2)	The SEAL LM server produces a requested spatial map using the processed spatial map data required to estimate the number of objects, type of object, position, direction, speed, etc.

Editor's Note: How the SEAL LM server gets the spatial map data for the three-dimensional area of interest is FFS.

3)	The SEAL LM server sends response message to the requestor with produced spatial map ID and information which includes three-dimensional space defined by the spatial map, stationary or moving objects with attributes of them.

Editor's Note: What else information elements are required as the spatial map information is FFS.

#### 7.8.3.2	Update spatial map

Figure 7.8.3.2-1 dipicsts the procedure to update the spatial map. For the request from the spatial map consumer (VAL server or SEAL LM client), modifying or deleting the spatial map is provided by SEAL LM server.

Figure 7.8.3.2-1: Update spatial map

1)	The VAL server (or SEAL LM client) sends a request message to the SEAL LM server to update a spatial map. The request includes requestor ID, spatial map ID, indicator to modify or delete, information on what to modify  such as updated access control rules, updated spatial map coverage area, etc. The SEAL LM server authorizes VAL server (or SEAL LM client) and validates the request.

Editor's Note: It is FFS which aspects of the relationship between spatial anchors and spatial maps are to be maintained and how. It can be done by the VAL layer, or by the service enablement layer.

2)	The SEAL LM server delete or modifies the spatial map as requested.

3)	Then it sends response message to the requestor with result of the request and modified spatial map information with the spatial map ID.

#### 7.8.3.3	Get spatial map

Figure 7.8.3.3-1 dipicts the procedure for the authorized spatial map consumer (VAL server or SEAL LM client) to get the spatial map. The service is provided by SEAL LM server.

Figure 7.8.3.3-1: Get spatial map

1)	The VAL server (or SEAL LM client) sends a request message to the SEAL LM server to get the spatial map. The request includes requestor ID, spatial map ID to get the information on it, three-dimensional area of interest or localization information to discover spatial maps in the area, optionally discovery filters, i.e. a set of characteristics to search the matching spatial maps when multiple spatial maps are allowed in the area of interest.

NOTE:	How the consumer identifies which three-dimensional area of interest to get the spatial map is not in the scope of this study.

Editor's Note: It is FFS whether multiple spatial maps are allowed in the given area of interest.

2)	The SEAL LM server authorizes VAL server (or SEAL LM client), and validates the request. Then the server collects the requested spatial map information. The SEAL LM server sends the get spatial map response message to the authorized third party consumer with spatial map ID and spatial map information when matching spatial map is found, otherwise it sends failure indication with reason.

#### 7.8.3.4	Subscribe/unsubscribe spatial map

Figure 7.8.3.4-1 dipicts the procedure for the authorized spatial map consumer (VAL server or SEAL LM client) to subscribe the spatial map. The service is provided by SEAL LM server.

Figure 7.8.3.4-1: Subscribe/unsubscribe spatial map

1)	The VAL server (or SEAL LM client) sends a request message to the SEAL LM server to subscribe/unsubscribe spatial map(s). The request includes requestor ID, spatial map ID(s) or three-dimensional area of interest (or localization information) to identify the applicable spatial map(s), event list to subscribe/unsubscribe. For the unsubscribe spatial map request, the subscription ID is included in the message.

2)	The SEAL LM server authorizes VAL server (or SEAL LM client), and validates the request. Then server add/delete the requestor into/from the subscriber list and event list of the spatial map, and sends response message with the result. For the subscribe spatial map request, the subscription ID is included in the message as well.

NOTE:	For the VAL server (or SEAL LM client) which unsubscribed all the events of the spatial map, the SEAL LM server removes the VAL server (or SEAL LM client) completely from the subscriber list of the spatial map. In this case, step 3) will not be initiated.

3)	When an event is detected, the SEAL LM server notifies the VAL servers (or SEAL LM clients) which subscribed the corresponding event. Spatial map notification message includes the information on the event, e.g. added or removed objects, position or direction changes of the objects in the spatial map etc.

Editor's Note: The events related to spatial map are FFS.

### 7.8.4	Solution evaluation

## Editor's Note:	This clause provides an evaluation of the solution addressing KI#4.

## 7.x	Solution #x: <title>

Editor's Note:	Provide a suitable title for the solution.

### 7.x.1	Solution description

Editor's Note:	This clause will describe the solution. Each solution should clearly describe which of the key issues it covers and how.

This solution maps to KI# a [and KI# b]. This solution …

### 7.x.2	Architecture Impacts

Editor's Note:	This clause provides the architecture impacts (if any) of the solution.

### 7.x.3	Corresponding APIs

Editor's Note:	This clause provides the corresponding APIs for supporting the solution.

### 7.x.4	Solution evaluation

Editor's Note:	This clause provides an evaluation of the solution addressing KI#x.

# 8	Deployment scenarios

## 8.1	General

Editor's Note:	This clause will provide a general description of the deployment scenarios.

## 8.x	Deployment model #x: <Title>

Editor's Note:	Provide a description of the deployment scenarios.

# 9	Business Relationships

Editor's Note:	Provide a description of the involved business relationships.

# 10	Overall evaluation

## 10.1	Architecture evaluations

Editor's Note:	This clause will provide evaluation of different architectures.

## 10.2	Key issue evaluations

Editor's Note:	This clause will provide evaluation of different solutions.

# 11	Conclusions

## 11.1	General conclusions

Editor's Note:	This clause will provide general conclusions for the study.

## 11.2	Conclusions of key issue #x

Editor's Note:	This clause will provide conclusions for the specific key issue.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2023-11 | SA6#58 | S6-233940 |  |  |  | FS_MetaApp - Skeleton for new TR | 0.0.0 |
| 2023-12 | SA6#58 | S6-234040 |  |  |  | FS_MetaApp - Scope | 0.1.0 |
| 2023-12 | SA6#58 | S6-234059 |  |  |  | Pseudo-CR on KI on Spatial anchor enabler | 0.1.0 |
| 2023-12 | SA6#58 | S6-234086 |  |  |  | KI on sensitive information exposure | 0.1.0 |
| 2024-02 | SA6#59 | S6-240631 |  |  |  | Digital avatars KI | 0.2.0 |
| 2024-02 | SA6#59 | S6-240638 |  |  |  | SA2 linkage to KI on User Sensitive Information | 0.2.0 |
| 2024-02 | SA6#59 | S6-240728 |  |  |  | Pseudo-CR on application enablement architecture for mobile metaverse services | 0.2.0 |
| 2024-02 | SA6#59 | S6-240732 |  |  |  | KI Spatial mapping | 0.2.0 |
| 2024-02 | SA6#59 | S6-240735 |  |  |  | Pseudo-CR on update of KI on exposure of user sensitive information | 0.2.0 |
| 2024-02 | SA6#59 | S6-240750 |  |  |  | Pseudo-CR on New solution on spatial anchor discovery | 0.2.0 |
| 2024-02 | SA6#59 | S6-240766 |  |  |  | KI on discovery and QoS control for metaverse services | 0.2.0 |
| 2024-02 | SA6#59 | S6-240775 |  |  |  | KI#1 Update | 0.2.0 |
| 2024-02 | SA6#59 | S6-240786 |  |  |  | Pseudo-CR on Solution for spatial anchors management | 0.2.0 |
| 2024-04 | SA6#60 | S6-241474 |  |  |  | _FS_Metaverse_App - definitions | 0.3.0 |
| 2024-04 | SA6#60 | S6-241589 |  |  |  | FS_Metaverse_App-New KI on PINAPP enhancement | 0.3.0 |
| 2024-04 | SA6#60 | S6-241590 |  |  |  | Pseudo-CR on Enhancements to EDGEAPP to support metaverse services | 0.3.0 |
| 2024-04 | SA6#60 | S6-241478 |  |  |  | update solution#2 | 0.3.0 |
| 2024-04 | SA6#60 | S6-241617 |  |  |  | Pseudo-CR on New solution for Spatial Anchor subscription | 0.3.0 |
| 2024-04 | SA6#60 | S6-241480 |  |  |  | Pseudo-CR on Solution #1 EN resolution | 0.3.0 |
| 2024-04 | SA6#60 | S6-241561 |  |  |  | Addition of New Study Aspects to the Key Issue#1 | 0.3.0 |
| 2024-04 | SA6#60 | S6-241625 |  |  |  | Enhancement to the update solution | 0.3.0 |
| 2024-04 | SA6#60 | S6-241562 |  |  |  | Update to spatial anchor discovery solution | 0.3.0 |
| 2024-04 | SA6#60 | S6-241624 |  |  |  | Update to the spatial anchor get solution | 0.3.0 |
| 2024-04 | SA6#60 | S6-241665 |  |  |  | New solution on the spatial anchor analytics related information | 0.3.0 |
| 2024-04 | SA6#60 | S6-241086 |  |  |  | KI on User Sensitive Information update | 0.3.0 |
| 2024-04 | SA6#60 | S6-241488 |  |  |  | FS_Metaverse_App-Update to KI 3 | 0.3.0 |
| 2024-04 | SA6#60 | S6-241645 |  |  |  | FS_Metaverse_App-Solution for digital avatar management | 0.3.0 |
| 2024-04 | SA6#60 | S6-241507 |  |  |  | Avatar support | 0.3.0 |
| 2024-04 | SA6#60 | S6-241661 |  |  |  | SEAL Architecture Option to support Digital Avatars | 0.3.0 |
| 2024-04 | SA6#60 | S6-241662 |  |  |  | Solution for KI#3 | 0.3.0 |
| 2024-04 | SA6#60 | S6-241638 |  |  |  | Pseudo-CR on Solution for Spatial mapping | 0.3.0 |