| 3GPP TR 23.700-23 V0.3.0 (2024-04) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Application enabler for XR Services; (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope		7

2	References		7

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms		8

3.2	Symbols		8

3.3	Abbreviations		8

4	Key issues		8

4.1	Key issue #1: KPI measurement and exposure	8

4.2	Key issue #2: E2E Multi-Modal Communication Flows	9

4.3	Key Issue #3: Coordination between direct UE connection and network based connection for AR/VR services		9

4.4	Key issue #4: Application enablement layer capabilities usage to supporting the XR services	9

4.5	Key issue #5: Support the tethered UE	10

4.6	Key issue #6: XR application server selection enhancement	10

4.7	Key issue #7: E2E KPI optimization for XR service	11

4.8	Key issue #8: PDU set handling for XR traffic	11

4.x	Key issue #x: <Title>	12

5	Application enablement architecture requirements	13

5.1	General requirements	13

5.2	Support for direct UE connection	13

5.2.1	Description	13

5.2.2	Requirements	13

5.3	Multi-modal flows alignment and monitoring requirements	13

5.3.1	Description	13

5.3.2	Requirements	13

5.X	<application layer capability x> requirements	13

6	Application enablement architecture for XR services	14

6.1	Application enablement architecture	14

6.1.1	General		14

6.1.2	Application enablement architecture based on SEALDD	14

6.2	Functional Elements	14

6.3	Reference Points	14

6.3.1	Mapping of multi-modal flows to reference points	14

7	Solutions		15

7.1	Mapping of solutions to key issues	15

7.1	Solution #1: SEALDD Policy Configuration for multi-modal flows	16

7.1.1	Architecture Impacts	16

7.1.2	Solution description	16

7.1.3	Information flows	16

7.1.4	Solution evaluation	17

7.2	Solution #2: Policy based Multi-modal SEALDD flow establishment	17

7.2.1	Architecture Impacts	17

7.2.2	Solution description	17

7.2.3	Solution evaluation	19

7.3	Solution #3: Support of Multi-Modal traffic indication in SEALDD layer	19

7.3.1	Architecture Impacts	19

7.3.2	Solution description	19

7.3.3	Solution evaluation	24

7.4	Solution #4: Support of QoS measurement for Multi-Modal traffic in SEALDD layer	24

7.4.1	Architecture Impacts	24

7.4.2	Solution description	24

7.4.3	Solution evaluation	32

7.5	Solution #5: Multi-flow synchronization for multi-modal XR application	32

7.5.1	Architecture Impacts	32

7.5.2	Solution description	32

7.5.3	Solution evaluation	34

7.6	Solution #6: Multi-modal flows alignment and monitoring	34

7.6.1	Architecture Impacts	34

7.6.2	Solution description	35

7.7	Solution #7: Support multi-modal service in SEALDD	38

7.7.1	Architecture Impacts	38

7.7.2	Solution description	38

7.7.3	Solution evaluation	44

7.8	Solution #8: EAS instantiation enhancement to satisfy E2E KPI requirements for XR application	45

7.8.1	Architecture Impacts	45

7.8.2	Solution description	45

7.8.3	Solution evaluation	46

7.x	Solution #x: <title>	46

7.x.1	Architecture Impacts	46

7.x.2	Solution description	47

7.x.3	Solution evaluation	47

8	Business Relationships	47

8.1	General		47

9	Overall evaluation	47

10	Conclusions		47

10.1	General conclusions	47

10.2	Conclusions of key issue #x	47

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

Introduction

XR (Extended Reality) refers to a set of general services which leverage High Data Rate Low Latency (HDRLL), AR/VR, and tactile/multi-modality communication.

SA1 defines the requirement for XR (Extended Reality) relevant Services. In Release-18, SA1 defined the requirement for tactile and multi-modal communication service in the TS 22.261 clause 6.43, including the support to provide policy(ies) for flows associated with an application, and apply 3rd party provided policy(ies).

XRM in SA2 studies the key issues, solutions and conclusions on the support of advanced media services, e.g. High Data Rate Low Latency (HDRLL) services, AR/VR/XR services, and tactile/multi-modality communication services. Also, some network information is going to be exposed, including the congestion level information, Data rate, delay difference and round trip delay of QoS flow.

Considering the XR requires high bandwidth and low latency requirement, it is important to explore how to ensure transmission quality from the application enabling layer, based on the capabilities of the 5GC and the policies or application information provided by AF(s).

1	Scope

The present document studies application enabling layer architecture requirements, key issues, and solution recommendations to support XR services.

The study is based on the requirements defined in TS 22.261 [2] and is dependent on the 5GC architecture supporting XRM services as specified in 3GPP TS 23.501 [3], 23.502 [4], and 23.503 [5].

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 22.261: "Service requirements for the 5G system".

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[4]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[5]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[6]	3GPP TS 23.433: "Service Enabler Architecture Layer for Verticals (SEAL);Data Delivery enabler for vertical applications;  Stage 2".

[7]	3GPP TR 26.998: "Support of 5G Glass-type Augmented Reality / Mixed Reality (AR/MR) devices".

[8]	3GPP TR 26.806: "Study on Tethering AR Glasses – Architectures, QoS and Media Aspects".

[9]	3GPP TS 26.522: "5G Real-time Media Transport Protocol Configurations".

[10]	3GPP TR 22.847: "Study on supporting tactile and multi-modality".

[11]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[12]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[13]	3GPP TR 22.856: "Feasibility Study on Localized Mobile Metaverse Services".

[14]	IETF RFC 6184: "RTP Payload Format for H.264 Video".

[15]	IETF RFC 7798: "RTP Payload Format for High Efficiency Video Coding (HEVC) ".

[16]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[17]	3GPP TS 28.538: "Management and orchestration; Edge Computing Management (ECM)".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

4	Key issues

4.1	Key issue #1: KPI measurement and exposure

According to clause 6.43 of TS 22.261 [2], synchronization between different media components is crucial for immersive multi-modal VR applications. This becomes particularly important when the synchronization threshold between two or more modalities is lower than the latency Key Performance Indicator (KPI) for the application, emphasizing the significance of minimizing delay differences. Otherwise, the arrived flows need to wait for other highly related flows that do not arrived yet. The SA2 provides the 5GS Packet Delay Variation monitoring to expose the variation of packet delay measured between UE and PSA UPF, which is only the single flow delay jitter. SA2 also defines the PDU Set Delay Budget (PSDB), which is the maximum delay that a PDU Set may experience for the transfer between the UE and the N6 termination point at the UPF. Application enabling layer could help to provide the ene-to-end measurement of the delay difference among those associated flows .

Another example is the duration of the burst. When the duration of the burst is long or the burst are too frequent, those indicators can suggest that the burst traffic may exceed the transmission capacity, leading to the viewer to detect frame dropping. Therefore, the duration of the burst, or, the frequency of burst may become an indicator that VAL may be interested in when providing XR services. The enabling layer can monitor the traffic to get the predicted duration of the burst based on the statistic, which can be used to do the traffic shaping.

To sum up, there are multiple indicators that may impact the performance of XR services, therefore this key issue will study:

-	identify what KPI the application enabling layer could provide that impact the QoS of the XR services.

NOTE: Coordination with SA4 and reuse of SA2 capabilities are needed.

-	how the KPI could be measured at the application enabling layer, and the enhancement of the SEALDD services.

4.2	Key issue #2: E2E Multi-Modal Communication Flows

Many XR use cases will require E2E multi-modal communication flows between application clients and application servers. SA1 has defined requirements for tactile and multi-modal communication service in 3GPP TS 22.261 clause 6.43, including the support to provide policy(ies) for E2E multi-modal communication flows associated with applications.

Using information provided by application clients and servers (e.g., policies), the application enablement layer, in coordination with the 5G CN, can support functionality to assist in managing E2E multi-modal communication flows between application clients and application servers.

This key issue will study:

-	Whether and how to support the E2E multi-modal communication flows between application clients and application servers within the application enablement layer?

-	Whether and how to support the interaction between the application enablement layer and 5G CN to manage E2E multi-modal communication flows between application clients and application servers?

-	Whether and how SEALDD may be enhanced to assist in managing E2E multi-modal communication flows between application clients and application servers involved in the same application service (e.g., support for multi-modal aware SEALDD flow management and policies)?

NOTE 1: This key issue should not impact 3GPP Core Network.

NOTE 2: SA6 should coordinate with SA4 on E2E multi-modal communication to ensure SA6 defined features are compatible with the needs of SA4. SA4 work will be reused for this KI as much as possible.

4.3	Key Issue #3: Coordination between direct UE connection and network based connection for AR/VR services

In clause 7.6.1 of 3GPP TS 22.261 [2], the gaming or training service data can be exchanged between two 5G connected AR/VR devices. Communication over a direct link between the UEs can improve performance and service experience. Communication over 5G network and direct wireless connection can be used in parallel between AR/VR devices for redundancy or for different kinds of multi-modal call flows. Based on user consent, operator policy and trusted 3rd party request, the 5G network shall support a means to authorize specific UEs to transmit data (e.g. AR/VR service data) via direct device connection in a certain location and time or when direct device connection cannot fulfil the required QoS, communication over 5G network can provide redundant connection.

It is required to study the following:

-	How the application enablement layer can be enhanced to support usage of direct communication between 5G connected AR/VR devices.

-	Whether and how the application enablement layer can be enhanced to make coordination (e.g., the connection redundancy) between network based communication and direct communication between AR/VR devices for AR/VR services communication.

4.4	Key issue #4: Application enablement layer capabilities usage to supporting the XR services

- The XRApp aims to help XR service providers in delivering immersive experience which is enabled by communicating and synchronizing data streams such that multiple participants have a consistent immersive impression.

- From SA6 perspective, in addition to the SEALDD, which could provide the data delivery services, more application enablement layer capabilities could be utilized.

Taking Online AR Gaming as an example, this gaming involves two types of users. Player UEs receive streams of the live game from the game server and send control signals back to the server, and spectators who can join the live game, receive the stream from various perspectives, and potentially interact with players through cheering or game reward mechanisms. To support online AR gaming, the following capabilities could be leveraged (with potential enhancements):

- Data delivery for guaranteed multi-modal flow transmission.

- Network Slice Capability Enablement function for Provisioning differentiated network slices for players and spectators;

- Location management function to facilitate geofencing for the designated game arena in AR gaming.

- AI/ML may be required for Image & Object Recognition.

Currently, the XRApp is identified to be deployed as functionality component of the SEALDD, but there is a gap between the current architecture and the above requirements. So it is proposed to study the following open issues:

-	Whether and how the identified application enablement capabilities could be utilized to jointly support the XR service.

-	Whether and how the application enablement architecture could support the XR service taking advantage of the identified application enablement capabilities.

4.5	Key issue #5: Support the tethered UE

Different from other types of AR UE, the end-to-end path for the tethered UE includes one more wireless/wireline tethering link between AR Glasses and the tethering 5G Phone. In order to fulfil the end-to-end QoS requirements for the AR glass session, the consumer needs to acquire the tethering link status via measurement and analysis, and takes it into account when determining the QoS for the 5G system link. With the tethering link status, the application enablement layer may communicate with AF for dynamic QoS policy adjustment accordingly.

The 5G WireLess Tethered AR UE is introduced in TR 26.998 [7], and is further studied in TR 26.806[8]. they provides the Segment-by-segment delay measurement solution based on the ICMP, and End-to-end delay measurement based on RTP. The End-to-end delay measurement relies on tethered device to do the RTP based measurement. While it successfully identifies non-5G delays, distinguishing delays from the tethered link and those after N6 remains challenging. The Segment-by-segment delay measurement utilizes the UPF and tethered device for ICMP based measurement, however how UPF retrieves the RTT between the UPF and the application server and further exposes the latency results to the AF are not supported in SA2 in current release. And all the measurement results are sent to the AF, indicating that SA4 does not play a role in optimizing latency based on the measurements.

The above study provides good foundation for this KI. This KI can further study how to identify tethered links and optimize transmission.

It is proposed to study the open issues:

-	Whether and how could the application enabling layer identify traffic flows from the tethered devices behind the UE.

-	Whether and how could the application enabling layer support the acquisition of tethering link status via measurement and analytic and provide a transporting optimization, considering different type of the tethered UE (stand along tethered device, or wire/wireless tethered device).

-	Whether and how the PINAPP could be utilized to support the tethered UE.

4.6	Key issue #6: XR application server selection enhancement

As for XR service, current XR application server selection mechanism using EDGEAPP may not be able to satisfy the strict latency requirement. And considering that XR application server may involve multi-model communication flow, to support multi-model communication flow synchronization, it is better to improve service experience to provide ensure the operation on the multi-model synchronization.

However, due to one DNAI may be mapped to multiple UPFs in different EDNs, thus only using DNAI to and application server may not be enough.

Consider the following case (as showed in figure 6.1):

The UE is in the overlapping area of EDN#1 and EDN#2, when performing service provisioning, the UE may anchor to UPF#2 however the UE may connect to the EES#1 for EAS information. Then the UE may connect to EAS#1 discovered from EES#1.

The issue may happen that the user plane path is not optimal, which may not be able to satisfy the XR application requirement.

Figure 4.6.1 scneario on XR application server selection

This key issue includes the following aspects:

- Whether and how enablement layer can support XR application server to further reduce service latency?

- Whether and how enablement layer can support XR application server selection?

NOTE: For this Key issue, the coordinate with SA2 may be required.

4.7	Key issue #7: E2E KPI optimization for XR service

In real-time XR service, E2E KPI (e.g. low latency) is critical to provide immersive experience for XR users. Typically, the overall processing of XR service consists of several parts, such as media rendering, media transmission over 5G network, etc. The XR application servers may have different capabilities to perform the intensive processing, such as media rendering, which results in differentiated processing delay. In other aspects, the traffic flow between XR application client and XR application may have the different transmission quality due to the network handling. The E2E performance of XR service is impacted not only by data transmission, but also by data processing.

Therefore, further study the E2E KPI optimization mechanism considering the processing delay of XR application server, to satisfy real-time XR service is required. For example, as the enable layer, how to discover or initiate the appropriate XR application server and/or how to interact with 5GS is critical to satisfy the E2E performance of XR service.

This key issue includes the following aspects:

- How to support the E2E KPI optimization to meet real-time XR service requirements, taking the processing delay of XR application server into account, e.g. considering the potential enhancements to EDGEAPP in XR application server discovery or instantiation.

- What available information from 5G network can be used by enable layer to optimize the real-time XR service requirements.

4.8	Key issue #8: PDU set handling for XR traffic

SA2 and SA4 introduced PDU set based handling to support XR traffic. As mentioned in 3GPP TS 23.501 [3] and 3GPP TS 26.522 [9], for multi-modal flows transported in different QoS flow, the importance of PDU set can be used to help RAN to discard PDUs belonging to the same PDU set across flows, when needed. The PDU set identification and corresponding set-based policy enforcement are performed in CN and RAN, and inclusion of PDU set information is done in application layer.

For XR DL data, PSA UPF can mark PDU set information in GTP based on received protocol extension, e.g., RTP extension. Then during AN congestion, 5G-RAN and/or UPF will perform specific packet handling (e.g. drop) for PDUs in a set and also for PDU sets in the same SDF based on PDU set information in GTP.

For XR UL data, UE lower layer can mark PDU set information in AN-protocol based on received protocol extension, e.g., RTP extension. Then during AN congestion, 5G-RAN will perform packet handling for PDUs in a set and also for PDU sets in the same SDF based on PDU set information in AN-protocol.

SEALDD was specified in 3GPP since Rel-18, the user plane protocol stack over 5GS is depicted in Figure 4.8-1. The Application content including SEALDD layer information and VAL payload can be transferred between the SEALDD client in the UE and the SEALDD server in the data network.

Figure 4.8-1: User plane protocol stack for SEALDD traffic transmission over 5GS

NOTE:	PDU layer corresponds to the PDU carried between the UE and the DN over the PDU Session. E.g. UDP/IP, TCP/IP, QUIC/UDP/IP.

Since the inclusion of PDU set information is done in application layer, the application enablement layer, in coordination with 5G CN, can also facilitate PDU set related handling for a single flow or multiple flows in XR traffic.

This key issue will study:

-	Whether and how SEALDD or other application enabler may be enhanced to facilitate XR communication flows between application clients and application servers involved in the same application service in relation to PDU set (e.g., support for PDU set handling, QoS measurement)?

4.x	Key issue #x: <Title>

Editor's Note:	Provide a suitable title for the key issue.

Editor's Note:	This subclause will describe the key issue.

5	Application enablement architecture requirements

5.1	General requirements

The general architecture requirements specified in clause 6.1 to 6.4 of 3GPP TS 23.433 [6] are applicable for XRApp service.

5.2	Support for direct UE connection

5.2.1	Description

This clause specifies the requirements related to support for coordination between direct UE connection and network based connection.

5.2.2	Requirements

[AR-5.2.2-a] The architecture shall support a mechanism to authorize specific UEs to transmit XR data via direct device connection.

[AR-5.2.2-b] The architecture shall support a mechanism for monitoring the availability and QoS of direct link between UEs for XR service data transmission.

[AR-5.2.2-c] The architecture shall be capable of reporting the availability and QoS of direct link for XR service data transmission.

[AR-5.2.2-d] The architecture shall be capable of making network based connections take over the XR data transmission when direct device connection cannot fulfil the required QoS or vice versa.

5.3	Multi-modal flows alignment and monitoring requirements

5.3.1	Description

This subclause specifies the requirements for multi-modal flows alignment and monitoring service.

5.3.2	Requirements

[AR-5.3.2-a] The XRApp shall provide a mechanism to enable multi-modal flows alignment by the authorized VAL server.

[AR-5.3.2-b] The XRApp shall provide a mechanism to enable multi-modal flows alignment monitoring by the authorized users or VAL server.

5.X	<application layer capability x> requirements

Editor's Note:	Provide a suitable title for the requirements.

Editor's Note: This subclause will describe the architectural requirements for the studied application layer capabilities.

6	Application enablement architecture for XR services

6.1	Application enablement architecture

6.1.1	General

This clause provides the overall architecture description.

6.1.2	Application enablement architecture based on SEALDD

To facilitate the specific optimization for XR application provided by 5G network, the application enablement architecture for the XRApp service is based on the generic functional model specified in 3GPP TS 23.434 [4] and 3GPP TS 23.433 [6].

The XRApp can be deployed as functionality component of the SEALDD, then the architecture for SEALDD is applicable to XRApp.

Editor’s Note: Whether a independent XRAPP architecture is needed is FFS, the gap between the XRApp and SEALDD needs to be identified.

Figure 6.1.2-1: Application enablement architecture based on SEALDD

6.2	Functional Elements

Editor's Note:	The functional elements corresponding to the architecture will be presented in this clause.

6.3	Reference Points

Editor's Note:	The reference points corresponding to the architecture will be presented in this clause.

6.3.1	Mapping of multi-modal flows to reference points

Figure 6.3.1-1 illustrates an example mapping of SEALDD multi-modal flows to the reference points of the application enablement architecture.

Multi-modal services are based on several data flows related to each other and subject to application coordination. The data flows can transfer different types of data (for example audio, video, positioning, haptic data) and may come from different sources(e.g. a single UE, a single device or multiple devices connected to the single UE, or multiple UEs). (e.g. 3GPP TR 23.501[3] clause 5.37.2

To support E2E multi-modal communication flows between multiple VAL clients and servers (e.g. 3GPP TR 22.847[10] clause 4.2), a SEALDD server and clients may support multi-modal service capabilities such as establishing and managing SEALDD multi-modal flows. Via SEALDD multi-modal flows, the SEALDD server and clients perform multi-modal traffic transfer and management processing (e.g., E2E synchronization of application traffic having multi-modal dependencies with one another).

Figure 6.3.1-1: Example SEALDD multi-modal flows mapping to reference points.

Figure 6.3.1-1 illustrates XR services using two different XR Servers (A and B). The services provided are used in one case by a single integrated VAL client, VAL client 3, on UE Y. The services provided are also used by UE X with VAL XR client 2 on UE X and VAL XR client 1 on a device tethered to UE X (see 3GPP TR 26.806[8] clause 4.4).

Editor’s Note: It is FFS whether the multi-modal flow usecase for tethered devices, as shown in the UE X example, is in scope of the current study.

7	Solutions

7.1	Mapping of solutions to key issues

Table 7.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 | KI #6 | KI #7 | KI #8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #1 |  | X |  |  |  |  |  |  |
| Sol #2 |  | X |  |  |  |  |  |  |
| Sol #3 |  | X |  |  |  |  |  |  |
| Sol #4 | X |  |  |  |  |  |  |  |
| Sol #5 |  | X |  |  |  |  |  |  |
| Sol #6 | X |  |  |  |  |  |  |  |
| Sol #7 |  |  |  |  |  |  |  | X |
| Sol #8 |  |  |  |  |  |  | X |  |

7.1	Solution #1: SEALDD Policy Configuration for multi-modal flows

7.1.1	Architecture Impacts

This solution adds support for multi-modal SEALDD policy configuration functionality by enhancing the existing SEALDD policy configuration functionality supported in clause 9.10 of TS 23.433[6]. There are no impacts to the  SEALDD architecture.

7.1.2	Solution description

This solution is targeting KI #2: E2E Multi-Modal Communication Flows and specifies enhancements to the SEALDD policy configuration information flows. In this procedure, the VAL server is a specific server for configuring multi-modal SEALDD policies and is different from VAL servers used for VAL application processing.

The SEALDD policy configuration procedure detailed in 3GPP TS23.433[6] clause 9.10.2.1 is reused with the enhanced information flows below

NOTE: Corresponding enhancements to multi-modal SEALDD policy update and delete procedures will be completed during the normative phase.

7.1.3	Information flows

7.1.3.1	SEALDD policy configuration request for multi-modal flows

Table 7.1.3.1-1 describes the new information elements to be added to the SEALDD policy configuration request in 3GPP TS 23.434[6] Table 9.10.3.1-1 in order to support multi-modal flows.

Table 7.1.3.1-1:  SEALDD policy configuration request IEs for multi-modal flows

| Information element | Status | Description |
| --- | --- | --- |
| Multi-modal VAL flows list | M | List of individual VAL flows applicable to this multi-modal SEALDD policy. |
| > Application traffic identifiers | M | Identifier of the application traffic for this individual VAL flow (e.g. VAL server ID, VAL service ID) |
| > VAL UE identity | O | Identifier of the VAL UE for this individual VAL flow |
| > SEALDD policy | M | SEALDD policy associated with this individual VAL flow as defined in Table 9.10.3.1-1 of TS 23.433 |
| Multi-modal SEALDD policy | M | Multi-modal SEALDD policy applicable to Multi-modal VAL flows list. This IE is used for complementing other SEALDD measurement policies for multi-modal flows |
| > Burst policy | O (See NOTE 1) | Policy used by the SEALDD layer to determine multi-modal flow configuration |
| > Synchronization policy | O (See NOTE 1) | Policy used by the SEALDD layer to perform multi-modal synchronization operations. |
| > Policy expiration time | O | Indicates the validity period of the policy |
| NOTE 1: At least one of these multi-modal SEALDD policies shall be present in the request |  |  |

NOTE 2: 	The burst policy and the synchronization policy, and their use, are to be harmonized with KI#1 solutions.

Editor’s Note:	How to harmonize the burst related parameters considering burst policy in DD policy is FFS.

7.1.4	Solution evaluation

Editor's note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

7.2	Solution #2: Policy based Multi-modal SEALDD flow establishment

7.2.1	Architecture Impacts

This solution describes policy-based multi-modal SEALDD flow establishment functionality.

7.2.2	Solution description

This solution is targeting KI #2: E2E Multi-Modal Communication Flows and specifies a policy based multi-modal SEALDD flow establishment procedure.

NOTE 1: In the normative phase, this solution is to be harmonized with the corresponding solution for multi-modal application data delivery initiated by VAL server, as well as with clause 9.2.2.3 of TS 23.433.

NOTE 2: A corresponding procedure for deletion of a multi-modal SEALDD flow is to be defined during the normative phase.

7.2.2.1	Multi-modal SEALDD flow establishment based on policy

Figure 7.2.2.1 illustrates the procedure for a SEALDD server to establish a multi-modal SEALDD flow based on a configured multi-modal SEALDD policy. The multi-modal SEALDD flow is established by the SEALDD server before multi-modal application communication between VAL clients and VAL servers starts.

Pre-conditions:

1.	The SEALDD server has been configured with a multi-modal SEALDD policy.

Figure 7.2.2.1-1: Policy based multi-modal SEALDD flow establishment

1.	The VAL servers subscribe to SEALDD event exposure for connection status using the procedure defined in TS 23.433[6] clause 9.2.2.6.

2.	Based on the configured multi-modal SEALDD policy information, the SEALDD server determines the applicable VAL flows and triggers the establishment of required individual SEALDD connections as described in TS 23.433 clause 9.2.2.3. Next, the SEALDD server triggers the establishment of a multi-modal SEALDD flow and associates the individual SEALDD connections with it. To maintain this association, the SEALDD server assigns a multi-modal SEALDD flow identifier and associates it with the identifiers of each individual SEALDD connection.

3.	During establishment of the multi-modal SEALDD flow, the SEALDD server interacts with the 3GPP CN to configure multi-modal QoS flow parameters based on the multi-modal SEALDD policy information. The SEALDD server utilizes the NEF/PCF/NRM/EES service for multi-modal QoS adjustment. Specifically, the SEALDD server relies on the northbound Policy Authorization Service API exposed by the PCF as specified in 3GPP TS 23.502 [4] and 3GPP TS 23.503 [5], if the SEALDD server is connected to the PCF via the N5 reference point, or the northbound AF Session with QoS Service API and/or the PFD Management northbound APIs exposed by the NEF as specified in 3GPP TS 23.502 [4] and 3GPP TS 23.503 [5], if the SEALDD server is connected to the PCF via NEF. SEALDD relies also upon the EES Session with QoS API as specified in 3GPP TS 23.558 [11] and/or the NRM QoS functionality as described in 3GPP TS 23.434 [12].

4.	The SEALDD server allocates an IP address(s) and port(s) for sending and receiving packet over the SEAL-S reference point. The SEALDD server then sends SEALDD connection establishment notification with VAL service ID, the IP address and port to each of the VAL servers.

5.	The SEALDD server allocates an IP address and port for sending and receiving packet over the SEAL-UU reference point. The SEALDD server then sends a regular data transmission connection establishment request to the SEALDD client with SEALDD flow ID, VAL service ID, the IP address and port and receives a response. UE IP address (and port) can be included by the SEALDD client in the response or sent in a separate update message by SEALDD client if a different UE IP address is to be used in SEALDD connection user plane.

NOTE 1:	Step 4 and step 5 can be done in parallel.

NOTE 2:	Step 5 can be sent via PDU session (if exist) or via application triggering (if no PDU session exists).

6.	The SEALDD client further notifies VAL clients about the SEALDD connections being established.

Upon receiving multi-modal application traffic from VAL clients (not shown in the figure), the SEALDD client sends it to SEALDD server in SEALDD traffic. The SEALDD server identifies multi-modal application traffic based on the VAL service ID and further sends the multi-modal application traffic to the VAL servers. The downlink multi-modal application traffic sent from VAL server to VAL client is processed similarly.

Editor’s Note: It is FFS whether this procedure applies to multiple XR VAL clients on the same SEAL-UU connection.

7.2.3	Solution evaluation

This solution enables end-to-end multi-modal connection between VAL client and VAL server for application data delivery, with the connection initiation being based on policy, and it is well-aligned with the existing functionality in clause 9.2.2.3 of TS 23.433. The solution has minimal architectural impacts.

7.3	Solution #3: Support of Multi-Modal traffic indication in SEALDD layer

7.3.1	Architecture Impacts

This solution is based on architecture of SEALDD as described in 3GPP TS 23.433 [6].

7.3.2	Solution description

7.3.2.1	General

This solution addresses the improvement of the SEALDD layer to support Multi-Modal service.

The solution improves the SEALDD layer defined in 3GPP TS 23.433, i.e.:

1.	The multiple flows can be requested in one SEALDD enabled regular data transmission connection establishment.

2.	The SEALDD server uses 5GC capabilities to configure XR traffic flows.

In this solution, the following definitions are used:

1.	Multi-Modal Service Identifier is an identifier of the multi-modal service as defined in clause 6.1.3.27.3 of 3GPP TS 23.503 [5].

2.	Multi-Modal Service Requirements is requirements for the multi-modal traffic handling in 5GC as defined in clause 6.1.3.27.3 of 3GPP TS 23.503 [5].

In this solution, the following assumptions are used:

1.	The VAL service identified by the VAL Service ID serves only one XR application. Thus, the VAL Service ID functionally equal to the Multi-Modal Service Identifier and may be generated by the SEALDD server based on VAL service ID and used by the SEALDD server for negotiation with 5GC via N33/N5 interfaces for the XR traffic handling.

2.	The VAL server(s) within one VAL Service may provide different functional parts of the XR application, e.g., VAL Server A performs video flow(s) processing, VAL Server B performs audio flow(s) processing, and/or VAL Server C preforms both video and audio flows processing. Thus, the Multi-Modal Service Requirements for the XR flows may be determined and generated by the SEALDD server based on VAL service ID and/or VAL server ID for negotiation with 5GC via N33/N5 interfaces for the XR traffic handling.

NOTE:	the Multi Model Service Identifier is not used in flow correlation.

7.3.2.2	Impact to existing SEALDD procedures

The SEALDD procedures and information flows in 3GPP TS 23.433 [6] can be enhanced (highlighted in bold italics) as follows.

NOTE:	SEALDD data transmission procedures with single SEALDD connection are described as typical example below.

9.2.2.2 SEALDD enabled regular data transmission connection establishment procedure Figure 9.2.2.2-1 illustrate the procedure for establishing regular SEALDD data transmission connection. Pre-condition: - The VAL server can discover and select the SEALDD server by CAPIF functions. Figure 9.2.2.2-1: SEALDD enabled regular data transmission connection establishment procedure 1. The VAL server decides to use SEALDD service for application traffic transfer and allocates address/port as SEALDD-S Data transmission connection information for receiving the data packets from SEALDD server. The VAL server sends Sdd_RegularTransmission request to the SEALDD server discovered by CAPIF. The service request includes UE ID/address, VAL server ID, VAL service ID, list of requested flows, including SEALDD-S Data transmission connection information of the VAL server side, and optionally, the QoS information for the application traffic, e.g. QoS requirements. 2. Upon receiving the request, the SEALDD server performs an authorization check. If authorization is successful, SEALDD server allocates address/port of the SEALDD server to receive the packets from the VAL server for application data transfer as SEALDD-S data transmission connection information of the SEALDD server side for each requested flow provided in step 1. The SEALDD server allocates a specific address or port used for SEALDD traffic transfer with the specific UE for the VAL server and responds with a SEALDD service response (including SEALDD-S data transmission connection information of the SEALDD server side) for each requested flow provided in step 1. The VAL server and SEALDD server can use SEALDD-S data transmission connection information to establish the data transmission connection between VAL server and SEALDD server for application data transfer. The SEALDD server may send the AF request to provide the required QoS information to 5GC via N33/N5 for each requested flow provided in step 1, as defined in clause 5.2.6.9 and in clause 5.2.5.3 of 3GPP TS 23.502 [6]. The AF request includes the application traffic descriptor containing the address or ports allocated by SEALDD server, and the QoS information for application traffic, and may include Multi-Modal Service ID together with Multi-modal Service Requirements. The QoS information may be determined by SEALDD server according to VAL service ID for different service type of application traffic if the QoS information is not provided by VAL server. The Multi-Modal Service ID may be determined by the SEALDD server according to VAL service ID. The Multi-modal Service Requirements may be determined by the SEALDD server according to VAL service ID and/or VAL server ID. The SEALDD server relies on the northbound Policy Authorization Service API exposed by the PCF as specified in 3GPP TS 23.502 [6] and 3GPP TS 23.503 [7], if the SEALDD server is connected to the PCF via the N5 reference point, or the northbound AF Session with QoS Service APIs and/or the PFD Management northbound APIs exposed by the NEF as specified in 3GPP TS 23.502 [6] and 3GPP TS 23.503 [7], if the SEALDD server is connected to the PCF via NEF. SEALDD may also rely upon the EES Session with QovS API as specified in 3GPP TS 23.558 [10] and/or the NRM QoS functionality as described in 3GPP TS 23.434 [4]. NOTE 1: The SEALDD-S data transmission connection information of the SEALDD server side is optional to respond to the VAL server, if the SEALDD server uses the downlink pull mode to obtain the data/content from the address provided by the VAL server in step 1, and uses the uplink push mode to send the data/content to the address provided by VAL server. 3. Data transmission sessions information is provisioned to the VAL client by the VAL server via application signalling. NOTE 2: The application signalling may be transmitted via direct application layer connection or via the SEALDD layer. 4. The VAL client sends a SEALDD service request to SEALDD client. The VAL client receives a SEALDD service response to the SEALDD client. The response indicates that whether the SEALDD service request is successful or not. 5. The VAL/SEALDD client discover and select the proper SEALDD server for the VAL application, as described in clause 9.4.3. After this step, the VAL server is discovered and selected along with the associated SEALDD server, the SEALDD client can get the SEALDD server's address. 6. The SEALDD client allocates a SEALDD flow ID mapping to the identifiers of the application traffic. The SEALDD client sends Sdd_RegularTransmissionConnection_Establish request to SEALDD server with the SEALDD client ID, the SEALDD flow ID, the SEALDD traffic descriptor of the SEALDD client side (the address/port of the SEALDD client for receiving the downlink SEALDD traffic), VAL server ID, VAL service ID. The request message also contains the selected VAL server endpoint information and UEID. The SEALDD server retrieves the location information of the VAL UE or SEALDD client from SEAL LM services defined in 3GPP TS 23.434[4] clause 9.3.12 and verifies with the Geofence policy configured by the VAL server to allow or restrict the data connection establishment. If the location information is allowed as per the configured geofence policy then the SEALDD server allows the data connection establishment, otherwise SEALDD server returns a failed result e.g. performs connection reject. NOTE 3: The SEALDD server can use or update the association between SEALDD-UU connection and SEALDD-S connection that associated with UE ID, VAL service ID, VAL server endpoint, which is used to correlate the SEALDD traffic and the VAL application traffic. NOTE 4: The SEALDD flow ID is used by the SEALDD client and SEALDD server to identify different VAL application traffic of the same SEALDD client. The SEALDD flow ID may be same with the identifiers of the application traffic or new simplified IDs allocated by SEALDD. 7. The SEALDD server responds to the SEALDD client with the SEALDD traffic descriptor of SEALDD server side (e.g. address/port allocated in step 2, transport layer protocol) mapping to the application traffic. 8. If the connection between VAL server and SEALDD server is not established in step 2, the SEALDD server establishes connection with VAL server for the VAL client to transmit application traffic mapping to the SEALDD traffic according to the SEALDD-S information negotiated in step 1-2. 9. The SEALDD client uses the SEALDD traffic descriptor of SEALDD server side for SEALDD connection establishment. After this step, the SEALDD client and SEALDD server both get the whole SEALDD traffic descriptor (including the UE's address/port and SEALDD server's address/port for the SEALDD traffic transmission). 10. If multiple requested flows are provided in step 1, the SEALDD client repeats steps 6-9 for each requested flow provided in step 1. NOTE 3: The way for the multiple flows configuration between SEALDD client and SEALDD server (i.e., re-use/modify Sdd_RegularTransmissionConnection_Establish procedure or use other new introduced procedure) will be decided in the normative stage. Editor’s Note: How to harmonize the burst related parameters (for PCF input) considering burst policy in DD policy is FFS. After the negotiation and establishment of the connections, the SEALDD client gets the mapping information between application traffic and SEALDD flow ID. The SEALDD server gets the mapping information between the SEALDD flow ID and the SEALDD-S connection. Upon receiving application traffic from VAL client, the SEALDD maps it to SEALDD traffic with SEALDD traffic descriptors as negotiated with SEALDD server in step 6 and step 7. The SEALDD traffic is sent to the SEALDD server. The SEALDD server maps the SEALDD traffic to the application traffic according to the stored SEALDD traffic descriptor, SEALDD client ID and SEALDD flow ID. The SEALDD server sends the recovered application traffic to the address provided by VAL server in step 1, via the connection established in step 2 or 8 according to the mapping information. The downlink application traffic sent from VAL server to VAL client is processed similarly. The SEALDD server receives any UE location change notification using SEAL LM services defined in 3GPP TS 23.434[4] clause 9.3.12, then the SEALDD server performs the data delivery in alignment with the geofence policy. If the UE is in the forbidden location or not allowed for the given VAL service to send/receive data as per the Geofence policy, then the SEALDD server performs action like releases the connection and informs the VAL server that UE is not reachable because in a forbidden location using connection event status procedure. If UE enters the allowed location area then the SEALDD server initiates the connection establishment using the procedure defined in clause 9.2.2.3. 9.2.3.1 SEALDD enabled regular transmission request Table 9.2.3.1-1 describes the information flow from the VAL server to the SEALDD server for requesting the regular application transmission service. Table 9.2.3.1-1: SEALDD enabled Regular transmission request Information element; Status; Description. VAL server ID; M; Identity of the VAL server. VAL service ID; O; Identity of the VAL service. UE identity; O; Identifier of specific UE, e.g. UE ID. List of requested flows; M; List of the requested multi-modal flows. > SEALDD-S Data transmission connection information; M; Address/port and/or URL of the VAL server to receive the application packets from the SEALDD server. > QoS information; O; QoS information provided by VAL server. > VAL server’s total bandwidth limit; O (See NOTE); The total bandwidth limit of VAL server, including UL/DL. > VAL users’ bandwidth limit; O (See NOTE); The bandwidth limits (i.e. minimum bandwidth requirement and maximum bandwidth limit) for VAL users, including UL/DL. NOTE: These IEs are used for the SEALDD enabled bandwidth control for different VAL users.. 9.2.3.2 SEALDD enabled regular transmission response Table 9.2.3.2-1 describes the information flow from the SEALDD server to the VAL server for responding to the regular application transmission. Table 9.2.3.2-1: SEALDD enabled regular transmission response Information element; Status; Description. Result; M; Success or failure.. List of SEALDD-S information Data transmission connection information; O; List of Address/port and/or URL of the SEALDD server to receive the packets from the VAL server for application traffic transfer (NOTE 2). Cause; O See NOTE; Indicates the reason for the failure, e.g. SEALDD policy mismatch.. NOTE 1: The IE is only present if the Result is failure. NOTE 2: The order of elements in this list matches the order of elements in the List of requested flows IE defined in table 9.2.3.1-1.

7.3.3	Solution evaluation

This solution addresses KI#2 for the following aspects:

-	Whether and how to support the interaction between the application enablement layer and 5G CN to manage E2E multi-modal communication flows between application clients and application servers?

-	Whether and how SEALDD may be enhanced to assist in managing E2E multi-modal communication flows between application clients and application servers involved in the same application service (e.g., support for multi-modal aware SEALDD flow management and policies)?

7.4	Solution #4: Support of QoS measurement for Multi-Modal traffic in SEALDD layer

7.4.1	Architecture Impacts

This solution is based on architecture of SEALDD as described in 3GPP TS 23.433 [6].

7.4.2	Solution description

The current QoS measurement in SEALDD layer supports the QoS measurements with VAL UE, VAL Group, or VAL UE identity list. These three options of the monitoring granularity are not able to fulfil the XR application scenario due to the XR application scanario requires the QoS measurement granularity per Multi-Modal flow and each VAL UE may have two or more active Multi-Modal flows at the same time.

The following definitions are used in the solution, i.e.:

1.	Traffic descriptor is an endpoint (e.g., Address/Port or URL) at the SEALDD server for the SEALDD traffic. The traffic descriptor uniquely identifies the flow and can be used for the flow identification in the QoS measurement request.

2.	Crossflow QoS measurement is a QoS measurement that considers: XR traffic route (e.g. {[traffic descriptor 1, UL]; [traffic descriptor 2, DL]}), and possible DL flows synchronization overhead.

NOTE:	The synchronization overhead includes latency, bitrate, etc.

The XR scenario requires finer granularity for the QoS measurements, crossflow QoS measurement and the mapping between QoS measurements and XR metrics, i.e.:

1.	The current QoS measurements in SEALDD layer supports end-to-end QoS measurements only; thus, an additional granularity for QoS measurements like UL/DL needs to be added for XR scenarios.

2.	The traffic descriptor (Address/port or URL at the SEALDD server) is proposed to use for the Multi-Modal flow identification.

3.	The crossflow QoS measurement provides added value for the XR service, for example, Motion to Photon can be estimated as crossflow RTT measurement (e.g. {[traffic descriptor 1, UL]; [traffic descriptor 2, DL]}). The detailed definition Motion-to-Photon is provided in clause 7.6.1 in 3GPP TS 22.856[13].

The solution improves the SEALDD layer defined in 3GPP TS 23.433, i.e.:

1.	The traffic descriptor(s) and crossflow measurement information are included into the SEALDD enabled data transmission quality measurement subscription request in order to signal the multi-modal service QoS request in SEALDD layer;

2.	Finer granularity of QoS measurement identifiers, i.e., UL/DL/E2E. Example: UL jitter.

3.	Improvement of the SEALDD Client for the crossflow RTT measurement for multiple devices in XR scenario, e.g., wireless controllers and headset interaction.

7.4.2.2	Impact to existing SEALDD procedures

The SEALDD procedures and information flows in 3GPP TS 23.433 [6] can be enhanced (highlighted in bold italics) as follows.

NOTE:	SEALDD data transmission procedures with single SEALDD connection are described as typical example below.

9.7.2.1 Data transmission quality measurement Figure 9.7.2-1 illustrate the procedure for SEALDD enabled data transmission quality measurement. The SEALDD client and SEALDD server is enhanced to carry out the data transmission quality measurement. Pre-conditions: 1. The SEALDD server and SEALDD client are synchronized to the time source provided by 5GS as specified in 3GPP TS 23.501 [5]. 2. The VAL server discovers and selects the SEALDD server by CAPIF functions. Figure 9.7.2.1-1: SEALDD enabled data transmission quality measurement procedure 1. An on-going regular data transmission connection is established according to clause 9.2.2.2. 2. The VAL server sends a SEALDD transmission quality measurement subscription request to the SEALDD server. The request includes the identifiers of the application traffic (e.g. VAL service ID, VAL server ID), requirement of transmission quality measurement (e.g. latency, jitter, bitrate, packet loss rate) and measurement target UE (e.g. a single UE, a group of UEs or all UEs), flow(s) traffic descriptor(s), and may also include reporting criteria, reporting frequency, spatial condition and temporal condition. NOTE: The spatial and/or temporal condition can be used by SEALDD server to apply when and where the measurement is performed. For instance, the measurement is expected to be done for a group of VAL UEs with a scheduled route (from city A to city B via highway A2 and A3), from 9:00 a.m. to 11:00 a.m. on Tuesday and from 1:00 p.m. to 5:00 p.m. on Thursday. 3. Upon receiving the request, the SEALDD server performs an authorization check. If authorization is successful, the SEALDD server sends a response to the VAL server with the subscription ID, expiration time. 4. If the transmission quality measurement requirement list provided by VAL server in step 2, indicates that the latency is needed to be measured, the SEALDD server initiates the DL packet delay measurement. The SEALDD server encapsulates the DL monitoring packet (i.e. DL SEALDD packet with SEALDD DL monitoring header and VAL traffic as payload, or dummy DL SEALDD packet generated for data transmission quality monitoring) with local time T1 when the SEALDD server sends out the DL monitoring packets. The SEALDD server considers the spatial and/or temporal conditions when starting/resuming the transmission quality measurement. If the conditions are not satisfied, the SEALDD server stops/suspends the transmission quality measurement. NOTE: For other metrics in transmission quality measurement requirement list (e.g. bitrate), the transmission quality result can be obtained by performance detection on the SEALDD server within a period of time. 5. The SEALDD client receives the DL monitoring packet, and records the local time T2. 6. Similarly, the SEALDD client encapsulates the UL monitoring packet (i.e. UL SEALDD packet with SEALDD UL monitoring header and VAL traffic as payload, or dummy UL SEALDD packet generated for data transmission quality monitoring) with local time T2 recorded in step 5 and local time T3 when the SEALDD client sends out the UL monitoring packet. 7. The SEALDD server records the local time T4 when the SEALDD server receives the UL monitoring packet and calculates the latency with T1, T2, T3, T4. The SEALDD server can also calculate the bitrate, jitter and packet loss rate over a certain period over a specific SEALDD connection by recording the status of the SEALDD packets carrying VAL traffic or dummy SEALDD packets generated for transmission quality measurement reports. The SEAL DD server also evaluates the reporting criteria if present in the SEALDD transmission quality measurement subscription request in order to generate the transmission quality measurement report. 8. The SEALDD server reports the data transmission quality measurement results (e.g. latency, jitter, bitrate, packet loss rate) to the VAL server via the notification message. When a VAL group ID or a list of VAL UE IDs or all VAL UEs indication is received in step 2, step 4 to step 7 is repeated for VAL UEs in the group/list or for all VAL UEs. The SEALDD server maps the VAL UE group ID to a list of VAL UE IDs if a VAL group ID is received. The SEALDD server identifies SEALDD connections corresponding to the desired VAL UE(s) or flow(s) traffic descriptor(s) to trigger measurement. And depending on the reporting requirement for multiple UEs, the SEALDD server calculates the needed report for the VAL server. When the VAL server decides to update or unsubscribe transmission quality measurement subscription after performing step 2 and step 3, the VAL server can send data transmission quality measurement subscription update request and data transmission quality measurement unsubscribe request to SEALDD server, as specified in Table 9.7.3.9-1 and Table 9.7.3.11-1, respectively. 9.7.2.2 Data transmission quality query Figure 9.7.2.2-1 illustrate the procedure for SEALDD enabled data transmission quality query. This procedure is used to obtain the historical transmission quality result already measured as described in clause 9.7.2.1. Pre-conditions: 1. The SEALDD server performs the data transmission quality measurement procedure, as described in clause 9.7.2.1. Figure 9.7.2.2-1: SEALDD enabled data transmission quality query procedure 1. The consumers (e.g. VAL server, SEALDD server, NSCE server, ADAE server) can send a SEALDD transmission quality query request to the SEALDD server to obtain the transmission quality measurement result. The request includes the identifiers of the application traffic (e.g. VAL service ID, VAL server ID), VAL UE ID or VAL UE group ID, flow(s) traffic descriptor(s), or crossflow measurement information. 2. The SEALDD server responds with the transmission quality measurement result (e.g. packet delay, bitrate, packet loss rate). 9.7.2.3 Data transmission quality measurement reported by SEALDD client Figure 9.7.2.3-1 illustrate the procedure for SEALDD enabled data transmission quality measurement for VAL traffic. The SEALDD client receives transmission quality measurement requirement, decides to start VAL data transmission monitoring and generates measurement reports. Figure 9.7.2.3-1: VAL data transmission quality measurement reported by SEALDD client 1. An on-going regular data transmission connection is established according to clause 9.2.2.2. The transmission quality measurement can be triggered by VAL server or VAL client, which is described in step 2 to step 5 and step 6, correspondingly. The request may include the crossflow measurement information (e.g. {[traffic descriptor 1, UL]; [traffic descriptor 2/DL]}) associated in the same multi-modal service for crossflow RTT measurement. 2. The VAL server sends a SEALDD transmission quality measurement subscription request to the SEALDD server. The request includes the identifiers of the application traffic (e.g. VAL service ID, VAL server ID), requirement of transmission quality measurement (e.g. latency, jitter, bitrate) and measurement target UE (e.g. a single UE, a group of UEs or all UEs), and may also include reporting criteria, reporting frequency, spatial condition and temporal condition. NOTE 1: The spatial and/or temporal condition can be used by SEALDD client to apply when and where the measurement is performed. For instance, the measurement is expected to be done for a group of VAL UEs with a scheduled route (from city A to city B via highway A2 and A3), from 9:00 a.m. to 11:00 a.m. on Tuesday and from 1:00 p.m. to 5:00 p.m. on Thursday. 3. Upon receiving the request, the SEALDD server performs an authorization check. If authorization is successful, the SEALDD server responds to the VAL server. 4-5. The SEALDD server sends a SEALDD transmission quality measurement subscription request to the SEALDD client and the SEALDD client responds to the SEALDD server. The SEALDD client, based on the received service quality guarantee policy including thresholds and action, can take corrective action as described in clause 9.7.2.3. If crossflow RTT measurement requirement is received from the VAL server, the SEALDD server identifies the SEALDD connections for the involved VAL UEs and sends to the SEALDD client(s) with crossflow RTT measurement requirement including corresponding UL/DL flow information. 6. The VAL client triggers the SEALDD transmission quality measurement procedure to the SEALDD client, in order to collect the measurement report information. 7. After SEALDD client determines to start measurement process, upon UL packet arrival, the SEALDD client initiates the UL packet delay measurement. The SEALDD client encapsulates the UL monitoring packet (i.e. UL SEALDD packet with SEALDD UL monitoring header and VAL traffic as payload for VAL data transmission quality monitoring) with local time T1 when the SEALDD client sends out the UL monitoring packet. The SEALDD client considers the spatial and/or temporal conditions when starting/resuming the transmission quality measurement. If the conditions are not satisfied, the SEALDD client stops/suspends the transmission quality measurement. For crossflow RTT measurement, the SEALDD client received UL flow information starts recording local time T1 when sending out the 1st UL packet (or the 1st UL packet in a PDU set) matching the UL flow information to the SEALDD server. The T1 is sent in the encapsulated UL monitoring packet to the SEALDD server. 8. The SEALDD server receives the UL monitoring packet, and records the local time T2. 9. Similarly, the SEALDD server encapsulates the DL monitoring packet (i.e. DL SEALDD packet with SEALDD DL monitoring header and VAL traffic as payload, or dummy UL SEALDD packet generated for data transmission quality monitoring in case there is no DL VAL traffic for DL packet delay monitoring) with local time T2 recorded in step 8 and local time T3 when the SEALDD server sends out the DL monitoring packet. NOTE 2: When the SEALDD server sends the dummy UL packet as monitoring response to the SEALDD client depends on SEALDD server implementation. For crossflow RTT measurement, when the 1st DL packet (or the last DL packet in a PDU set) matching the received DL flow information is to be sent, the SEALDD server encapsulates the DL monitoring packet with previously received T1 and sends the DL monitoring packet to the SEALDD client. 10. The SEALDD client records the local time T4 when the SEALDD client receives the DL monitoring packet and calculates the latency with T1, T2, T3, T4. The SEALDD client can also calculate the bitrate and jitter over a certain period over a specific SEALDD connection by recording the status of the SEALDD monitoring packets. The SEALDD client also evaluates the reporting criteria if present in the SEALDD transmission quality measurement subscription request in order to generate the transmission quality measurement report. For crossflow RTT measurement, if T1 is received in the DL packet, the SEALDD client records local time T2 and calculates RTT based on T1 and T2. Depending on which entity triggers the data transmission quality measurement, step 11 and step 12 corresponds to step 2 to step 5, step 13 corresponds to step 6. 11-12. The SEALDD client reports the data transmission quality measurement results (e.g. latency, jitter, bitrate, crossflow QoS measurements) to the VAL server via the SEALDD server. 13. The SEALDD client reports the data transmission quality measurement results to the VAL client. NOTE 3: The crossflow RTT measurement calculated by the SEALDD client in step 10 can be used as an estimation of the lower bound for the Motion-to-Photon latency in the XR scenarios. When a VAL group ID or a list of VAL UE IDs or all VAL UEs indication is received in step 2, step 4 to step 11 is repeated for VAL UEs in the group/list or for all VAL UEs. The SEALDD server maps the VAL UE group ID to a list of VAL UE IDs if a VAL group ID is received. The SEALDD server identifies SEALDD connections corresponding to the desired VAL UE(s) to trigger measurement. And depending on the reporting requirement for multiple UEs, the SEALDD server collects and aggregates the needed report for the VAL server. 9.7.3.1 SEALDD enabled data transmission quality measurement subscription request Table 9.7.3.1-1 describes the information flow from the VAL server to the SEALDD server for subscribing to the data transmission measurement service. Table 9.7.3.1-1: SEALDD transmission quality measurement subscription request Information element; Status; Description. Application traffic identifiers; M; Identify of the application traffic (e.g. VAL server ID, VAL service ID). VAL UE identity; O (See NOTE); Identifier of the VAL UE for which measurements need to be provided, e.g. UE ID. VAL UE group ID; O (See NOTE); Identifier of a specific VAL UE group, as defined in clause 7.5 of 3GPP TS 23.434 [4].. VAL UE identity list; O (See NOTE); Identifies a list of VAL UEs, e.g. the list of UE ID.. All VAL UEs Indication; O (See NOTE); Indicates all VAL UEs of the application identified by application traffic identifiers.. Flow(s) traffic descriptor(s); O (See NOTE); Indicates the flow(s) traffic descriptor(s) (Address/port or URL) for the measurement.. Crossflow measurement information; O (see NOTE); Represents the crossflow measurement information, e.g., list of pairs traffic descriptor with traffic direction (UL or DL). Example of the crossflow measurement information: {[traffic descriptor 1, UL]; [traffic descriptor 2, DL]}.. Measurement conditions; O; Indicates the temporal and/or spatial conditions.. Transmission quality measurement requirements list; M; The measurement requirement information. > Measurement ID; M; Measurement identifiers, e.g. UL/DL/E2E latency, UL/DL bitrate, UL/DL/E2E packet loss rate, UL/DL/E2E jitter, and crossflows identifiers (e.g., crossflows latency, crossflows bitrate, crossflows packet loss rate, crossflows jitter).. > Reporting frequency; O; The reporting frequency of measurement results (e.g. periodic reporting). If not present, it implies periodic reporting.. > Reporting periodicity; O; If the reporting frequency is periodic, the reporting periodicity shall be provided. For multiple UEs, it is recommended to give sufficient time to allow report aggregation.. > Reporting granularity; O; The reporting granularity indicates whether the measurement report is for individual VAL UE or for VAL UE group or for all VAL UEs, if VAL UE group or all VAL UEs is the measurement target.. > Measurement period window; O; Indicates the measurement period window for transmission quality measurements. > Measurement expiration time; O; Indicates the measurement expiration time. > Reporting criteria; O; Indicates the criteria for reporting measurement results, e.g. if the latency or bitrate reaches below or above a certain value. It also includes a unique identifier for each criteria of more than one criteria is specified.. NOTE: One of them shall be present as the measurement target UE or traffic descriptor.. 9.7.3.3 SEALDD enabled data transmission quality measurement notification Table 9.7.3.3-1 describes the information flow from the SEALDD server to the VAL server for notifying the transmission quality measurement reports. Table 9.7.3.3-1: SEALDD transmission quality measurement notification Information element; Status; Description. Subscription ID; M; Subscription identifier corresponding to the subscription.. Transmission quality measurement reports list; M; The generated transmission quality results in SEALDD server, as specified in Table 9.7.3.3-2.. Table 9.7.3.3-2 describes the information elements for the transmission quality measurement reports list, provided by the SEALDD server after performing transmission quality measurement. Table 9.7.3.3-2: SEALDD transmission quality measurement reports list Information element; Status; Description. > Measurement ID; M; Measurement identifiers (e.g. UL/DL/E2E latency, UL/DL bitrate, UL/DL/E2E packet loss rate, UL/DL/E2E jitter) and crossflows identifiers (e.g., crossflows latency, crossflows bitrate, crossflows packet loss rate, crossflows jitter). > VAL UE ID(s) or flow(s) traffic descriptor(s); O (see NOTE); It indicates the VAL UE(s) under SEALDD measurement. For a single VAL UE or flow(s) traffic descriptor(s), it can be omitted and the associated measurement values are for the single VAL UE. For multiple VAL UEs or flow(s) traffic descriptor(s) with reporting granularity set to individual UE or flow traffic descriptor, the associated measurement values are for individual VAL UE or flow traffic descriptor as indicated in this IE. For multiple VAL UEs with reporting granularity set to VAL UE group/list or all VAL UEs, the associated measurement values are aggregation for all VAL UEs or the VAL UE group/list and this IE includes the measured VAL UEs.. > Crossflow measurement information; O (see NOTE); Represents the crossflow measurement information, e.g., list of pairs traffic descriptor with traffic direction (UL or DL). Example of the crossflow measurement information: {[traffic descriptor 1, UL]; [traffic descriptor 2, DL]}.. > Average measurement value; O; The average measurement value of measurement results. > Minimum measurement value; O; The minimum measurement value of measurement results. > maximum measurement value; O; The maximum measurement value of measurement results. > Standard deviation measurement value; O; Standard deviation measurement value of measurement results. > kPercentile measurement value; O; Indicates the kpercentile measurement value of measurement results. > Measurement period; O; Indicates the measurement period. > Timestamp; O; Indicates the timestamp of measurement results. NOTE: These IEs are mutually exclusive.. 9.7.3.4 SEALDD enabled data transmission quality query request Table 9.7.3.4-1 describes the information flow from the other consumers (e.g. SEALDD server, NSCE server, ADAE server) to the SEALDD server for querying the data transmission quality measurement result. Table 9.7.3.4-1: SEALDD transmission quality query request Information element; Status; Description. Application traffic identifiers; M; Identify of the application traffic (e.g. VAL server ID, VAL service ID). VAL UE ID(s) or flow(s) traffic descriptor(s); O (see NOTE); Identifier of VAL UE(s) or flow (s) traffic descriptor(s) need to be queried, e.g. single VAL UE, multiple VAL UEs, flow(s) traffic descriptor(s), or VAL UE group. Crossflow measurement information; O (see NOTE); Represents the crossflow measurement information, e.g., list of pairs traffic descriptor and traffic direction (UL or DL). Example of the crossflow measurement information: {[traffic descriptor 1, UL]; [traffic descriptor 2, DL]}.. NOTE: These IEs are mutually exclusive..

7.4.3	Solution evaluation

This solution addresses KI#1 to support:

-	what KPI the application enabling layer could provide that impact the QoS of the XR services.

-	how the KPI could be measured at the application enabling layer, and the enhancement of the SEALDD services.

7.5	Solution #5: Multi-flow synchronization for multi-modal XR application

7.5.1	Architecture Impacts

This solution is presented based on the existing SEALDD architecture in TS 23.433 [6].

7.5.2	Solution description

7.5.2.1	General

This solution addresses key issue #2 on E2E multi-modal communication flow, to solve how the SEALDD layer to manage the multiple association flows for multi-modal XR service to satisfy the multi-flow synchronization.

By performing the transmission quality measurements for multi-modal application, the SEALDD server can determine the service flow that needs to be adjusted and the corresponding required QoS information based on the transmission quality result and the synchronization threshold.

7.5.2.2	Procedure

The following figure provides a high-level procedure that the SEALDD server determines which flow(s) is needed to be adjusted for multi-modal application, and interacts with 5GC with the determined QoS information to adjust the transmission quality to satisfy multi-flow synchronization.

Pre-conditions:

1.	The VAL server discovers and selects the SEALDD server by CAPIF functions.

2.	The SEALDD server has been provisioned to multi-modal XR policy including the synchronization threshold for multi-modal applictaion, as specified in TS 22.261 [2].

Figure 7.5.2.2-1: SEALDD enabled multi-flow synchronization procedure

- The VAL server sends a SEALDD Transmission request for multi-modal XR to the SEALDD server. The request includes the identifier of the application traffic (e.g. VAL service ID, VAL server ID), the SEALDD-S connection information for multiple flows from VAL server side (e.g. address/port for audio flow, address/port for tactile flow, etc).

- The SEALDD server determines to enable data transmission and synchronization for the multi-modal application (e.g. VAL service ID, VAL server ID). The SEALDD server allocates the SEALDD-S connection information for multiple flows from SEALDD server side, and maintains the multi-flow association information for SEALDD-S interface (i.e., SEALDD-S connection information from VAL server side, and/or SEALDD server side), and sends a response to the VAL server.

NOTE 1: The detailed message/API name for SEALDD transmission request for multi-modal XR will be determined in the normative phase.

- The VAL client sends a SEALDD service request to SEALDD client.

- The VAL/SEALDD client discover and select the proper SEALDD server for the VAL application.

- The SEALDD client sends SEALDD Transmission Connection_Establish request for multi-modal XR to SEALDD server with the SEALDD client ID, VAL server ID, VAL service ID, and the SEALDD traffic descriptor of the SEALDD client side.

- According to the determination in step 2, if the data transmission and synchronization is enabled for the specified applictaion identifier (e.g. VAL service ID, VAL server ID), the SEALDD server sends the SEALDD traffic descriptors for multiple flows from SEALDD server side (e.g. address/port for multiple SEALDD-Uu flow) to the SEALDD client. The SEALDD server maintains the multi-flow association information for SEALDD-Uu interface (i.e., SEALDD traffic descriptor from SEALDD client side, and/or SEALDD server side).

NOTE 2: Whether negotiating the SEALDD-Uu connection information for multi-modal XR, by using one control plane message or mutiple message control plane messages, and the related message/API name will be determined in the normative phase.

- The SEALDD server performs data transmission quality measurement in SEALDD-Uu interface based on the mapping information for multiple flow association information between SEALDD-S interface and SEALDD-Uu interface. Upon receiving the packets from multiple associated flow in SEALDD-S interface as descripted in step 2, the SEALDD server performs the packet encapsulation with sending timestamp information in the corresponding SEALDD-Uu interface, and can calculate the transmission delay measurement result of multiple associated flows after obtaining the receiving timestamp from the SEALDD client.

- Based on the calculated transmission delay for multiple associated flow over SEALDD-Uu interface in step 7, and the synchronization threshold for multi-modal application as described in pre-condition, the SEALDD server can determine the service flow(s) (i.e. address/port for SEALDD-Uu flow) that needs to be adjusted among the multiple associated flows in SEALDD-Uu interface, and the corresponding required QoS information (i.e. transmission delay).

- The SEALDD server sends the AF request to 5GC via N33/N5 with the SEALDD traffic descriptor of the adjusted flow(s) (i.e. address/port for the adjusted SEALDD-Uu flow) and the corresponding required QoS information determined in step 8, by utilizing the AF session with required QoS procedure in clause 4.15.6.6 of TS 23.502 [4]. The SEALDD traffic descriptor of the adjusted flow(s) contains the address or port in SEALDD server side, and/or SEALDD client side.

NOTE 3: This procedure is applicable for both downlink and uplink synchronization of multi-modal flow. For downlink and/or uplink, the step 8 is determined according to the measured downlink and/or uplink transmission delay in step 7.

After requesting the transmission quality optimization on 5G network with the required QoS for the adjusted flow(s), the multi-flow synchronization of multi-modal application can be satisfied.

7.5.3	Solution evaluation

Editor's note:	Solution evaluation is FFS.

7.6	Solution #6: Multi-modal flows alignment and monitoring

7.6.1	Architecture Impacts

- This solution requires a mechanism for data storage to cache the traffic flows.

7.6.2	Solution description

7.6.2.1	General

- Synchronization between different multi-modal XR application components is crucial. In other words, the delay difference among the associated flows should be small. Otherwise, the arrived flows need to wait for other associated flows that do not arrived yet.

- To avoid the downlink traffic flow delay difference introduced from (R)AN and Internet, the SEALDD/XRApp client could do the multi-modal traffic flow alignment, by steering associated multi-modal traffic flows to be transferred to application client at specific time.

- As shown in figure 7.6.2.1-1, flow#1 and flow#2 are associated flows. They are sent out from VAL server at the same time, while flow#1 arrives at SEALDD/XRApp client earlier than flow#2 due to different routing path. To align those associated flows, the SEALDD/XRApp client holds and buffers the flow#1 for a while. Until all the associated flows arrived(i.e., flow#2), the SEALDD/XRApp client sends out those associated flows to application client at the same time.

Editor’s note: Whether the implementation entity is XRApp or SELADD depends on the the Application enablement architecture.

Figure 7.6.2.1-1: DL traffic flow alignment

7.6.2.2	Procedure of flow alignment

- Figure 7.6.2.2-1illustrates the procedures of multi-modal flows alignment.

- Precondition:

-	The VAL server can discover and select the XRApp server by CAPIF functions.

-	The connection has been established among the UE, XRApp server and VAL server.

SEALDD/XRApp client

5GC

SEALDD/XRApp server

1. multi-modal flows coordination request

VAL server

5. flows coordination

4. multi-modal flows coordination response

connection established

2. multi-modal flows coordination request

3. multi-modal flows coordination response

Application client

Figure 7.6.2.2-1: multi-modal flows alignment

- VAL server sends the multi-modal flows alignment request with VAL ID, UE ID(s), flows description (e.g., Multi-modal Service ID, packet filter description, etc.), flows transmission requirement, flows alignment assistance information. Flows transmission requirement including the delay requirement. Flows alignment assistance information may include the timestamp (e.g., timestamp in the RTP header), number of associated flows, maximum acceptable duration for traffic flow alignment. Maximum acceptable time duration for traffic flow alignment is used to limit the maximum waiting time for the associated flow.

- Upon receiving the request, the SEALDD/XRApp server performs an authorization check. If authorization is successful, the SEALDD/XRApp server sends the multi-modal flows alignment request to the SEALDD/XRApp client identified by the UE ID(s), along with the information received in step1.

If the maximum acceptable duration for traffic flow alignment is not provided, then the XRApp server would determine the maximum acceptable duration for traffic flow alignment based on flows transmission requirement. The server may translate the flows description into SEALDD flow ID.

- The SEALDD/XRApp client sends the multi-modal flows alignment response to the SEALDD/XRApp server.

- The SEALDD/XRApp server sends the multi-modal flows alignment response to the VAL server indicating the result of multi-modal flows alignment.

- When the traffic flow arrived, the SEALDD/XRApp client initiate the multi-modal flows alignment based on the received information. The flows needs to be aligned is detected by the VAL ID, flows description. If number of associated flows are provided, the number of associated flows are used to determine whether all the associated flows arrived. If the timestamp is provided, the flows that are sent at specific time will be aligned. After all associated flows arrived, the SEALDD/XRApp client sends the flows to the application client. If the maximum acceptable time duration is provided, once the maximum acceptable time is reached, the SEALDD/XRApp client will no longer waits for the associated flows even if the associated flow is not arrived yet.

NOTE: Step3 and step5 can be executed in parallel.

7.6.2.3	Procedure of flow alignment monitoring

- Figure 7.6.2.3-1illustrates the procedure of multi-modal flows alignment monitoring.

- Precondition:

SEALDD/XRApp client

5GC

SEALDD/XRApp server

1. multi-modal flows coordination monitoring request

VAL server

5. flows coordination monitoring

7. multi-modal flows coordination monitoring notification

multi-modal flows alignment

2. multi-modal flows coordination monitoring request

4. multi-modal flows coordination monitoring response

6. multi-modal flows coordination monitoring notification

3. multi-modal flows coordination monitoring response

-	The multi-modal flows alignment has been requested and triggered.

Figure 7.6.2.3-1: traffic flow alignment monitoring

- VAL server sends the multi-modal flows alignment monitoring request. VAL ID, UE ID, flows description, monitoring period are provided in the request.

- Upon receiving the request, the SEALDD/XRApp server performs an authorization check. If authorization is successful, the SEALDD/XRApp server sends the multi-modal flows alignment monitoring request to the SEALDD/XRApp client.

- The SEALDD/XRApp client sends the multi-modal flows alignment monitoring response to the SEALDD/XRApp server.

4.	The SEALDD/XRApp server sends the multi-modal flows alignment monitoring response to the VAL server, indicating whether the multi-modal flows alignment monitoring request is successful.

5.	The XRApp client calculates the end-to-end delay difference based on the arrival time and timestamp(e.g., timestamp in the RTP header) among the flows.

Editor’s note: The detail of the end-to-end delay difference calculation is FFS.

6-7.XRApp client sends the multi-modal flows alignment monitoring result to the XRApp server, then to the VAL server.

7.7	Solution #7: Support multi-modal service in SEALDD

7.7.1	Architecture Impacts

This solution is based on architecture of SEALDD as described in 3GPP TS 23.433 [6].

7.7.2	Solution description

7.7.2.1	General

This solution addesses the improvement of the SEALDD layer to support Multi-Modal service.

In order to simplify VAL handling, the SEALDD layer can perform packetilization. Figure 7.7.2.1-2 illustrates packetilization in SEALDD layer and information flows:

Figure 7.7.2.1-2: SEALDD packetilization for VAL media data

The VAL can send media data (e.g. H.264 raw data in the form of [NALU][NALU]…[NALU]) to the SEALDD layer. After SEALDD packetilization using RTP (e.g. for H.264 as in IETF RFC 6184 [14] or H.265 as in IETF RFC 7798 [15]), NAL units can be segmented (if size is too large for line transmission) or aggregated (for small NAL units) by SEALDD, SEALDD also updates NALU header. Between the SEALDD client and SEALDD server, the SEALDD payload transferred via SEALDD connection (e.g. SEALDD/UDP/IP) includes RTP/RTCP/RTSP data.

NOTE:	The media data between SEALDD server and VAL server can be transferred via websocket which is implementation specific.

7.7.2.2	Impact to existing SEALDD procedures

The SEALDD procedures and information flows in 3GPP TS 23.433 [6] can be enhanced (highlighted in bold italics) as follows.

NOTE:	SEALDD data transmission procedures with single SEALDD connection are described as typical example below.

9.2.2.2 SEALDD enabled regular data transmission connection establishment procedure Figure 9.2.2.2-1 illustrate the procedure for establishing regular SEALDD data transmission connection. Pre-condition: - The VAL server can discover and select the SEALDD server by CAPIF functions. Figure 9.2.2.2-1: SEALDD enabled regular data transmission connection establishment procedure 1. The VAL server decides to use SEALDD service for application traffic transfer and allocates address/port as SEALDD-S Data transmission connection information for receiving the data packets from SEALDD server. The VAL server sends Sdd_RegularTransmission request to the SEALDD server discovered by CAPIF. The service request includes UE ID/address, VAL server ID, VAL service ID, SEALDD-S Data transmission connection information of the VAL server side, and optionally, the protocol description, the QoS information for the application traffic, e.g. QoS requirements. 2. Upon receiving the request, the SEALDD server performs an authorization check. If authorization is successful, SEALDD server allocates address/port of the SEALDD server to receive the packets from the VAL server for application data transfer as SEALDD-S data transmission connection information of the SEALDD server side. The SEALDD server allocates a specific address or port used for SEALDD traffic transfer with the specific UE for the VAL server and responds with a SEALDD service response (including SEALDD-S data transmission connection information of the SEALDD server side). The VAL server and SEALDD server can use SEALDD-S data transmission connection information to establish the data transmission connection between VAL server and SEALDD server for application data transfer. The SEALDD server may send the AF request to provide the required QoS information to 5GC via N33/N5, as defined in clause 5.2.6.9 and in clause 5.2.5.3 of 3GPP TS 23.502 [6]. The AF request includes the application traffic descriptor containing the address or ports allocated by SEALDD server, and the QoS information for application traffic. The QoS information may be determined by SEALDD server according to VAL service ID for different service type of application traffic if the QoS information is not provided by VAL server. The SEALDD server relies on the northbound Policy Authorization Service API exposed by the PCF as specified in 3GPP TS 23.502 [6] and 3GPP TS 23.503 [7], if the SEALDD server is connected to the PCF via the N5 reference point, or the northbound AF Session with QoS Service APIs and/or the PFD Management northbound APIs exposed by the NEF as specified in 3GPP TS 23.502 [6] and 3GPP TS 23.503 [7], if the SEALDD server is connected to the PCF via NEF. SEALDD may also rely upon the EES Session with QoS API as specified in 3GPP TS 23.558 [10] and/or the NRM QoS functionality as described in 3GPP TS 23.434 [4]. For application multi-modal service, the SEALDD server derives PDU Set related assistance information based on received VAL service ID and/or VAL server ID, and protocol description for interacting with NEF/PCF. NOTE 1: The SEALDD-S data transmission connection information of the SEALDD server side is optional to respond to the VAL server, if the SEALDD server uses the downlink pull mode to obtain the data/content from the address provided by the VAL server in step 1, and uses the uplink push mode to send the data/content to the address provided by VAL server. 3. Data transmission session information is provisioned to the VAL client by the VAL server via application signalling. NOTE 2: The application signalling may be transmitted via direct application layer connection or via the SEALDD layer. 4. The VAL client sends a SEALDD service request to SEALDD client. The VAL client receives a SEALDD service response to the SEALDD client. The response indicates that whether the SEALDD service request is successful or not. 5. The VAL/SEALDD client discover and select the proper SEALDD server for the VAL application, as described in clause 9.4.3. After this step, the VAL server is discovered and selected along with the associated SEALDD server, the SEALDD client can get the SEALDD server's address. 6. The SEALDD client allocates a SEALDD flow ID mapping to the identifiers of the application traffic. The SEALDD client sends Sdd_RegularTransmissionConnection_Establish request to SEALDD server with the SEALDD client ID, the SEALDD flow ID, the SEALDD traffic descriptor of the SEALDD client side (the address/port of the SEALDD client for receiving the downlink SEALDD traffic), VAL server ID, VAL service ID. The request message also contains the selected VAL server endpoint information and UEID. The SEALDD server retrieves the location information of the VAL UE or SEALDD client from SEAL LM services defined in 3GPP TS 23.434[4] clause 9.3.12 and verifies with the Geofence policy configured by the VAL server to allow or restrict the data connection establishment. If the location information is allowed as per the configured geofence policy then the SEALDD server allows the data connection establishment, otherwise SEALDD server returns a failed result e.g. performs connection reject. NOTE 3: The SEALDD server can use or update the association between SEALDD-UU connection and SEALDD-S connection that associated with UE ID, VAL service ID, VAL server endpoint, which is used to correlate the SEALDD traffic and the VAL application traffic. NOTE 4: The SEALDD flow ID is used by the SEALDD client and SEALDD server to identify different VAL application traffic of the same SEALDD client. The SEALDD flow ID may be same with the identifiers of the application traffic or new simplified IDs allocated by SEALDD. 7. The SEALDD server responds to the SEALDD client with the SEALDD traffic descriptor of SEALDD server side (e.g. address/port allocated in step 2, transport layer protocol) mapping to the application traffic. The SEALDD server may send the protocol description (UL related info) received from the VAL server to the SEALDD client in the SEALDD regular transmission connection establishment response. 8. If the connection between VAL server and SEALDD server is not established in step 2, the SEALDD server establishes connection with VAL server for the VAL client to transmit application traffic mapping to the SEALDD traffic according to the SEALDD-S information negotiated in step 1-2. 9. The SEALDD client uses the SEALDD traffic descriptor of SEALDD server side for SEALDD connection establishment. After this step, the SEALDD client and SEALDD server both get the whole SEALDD traffic descriptor (including the UE's address/port and SEALDD server's address/port for the SEALDD traffic transmission). After the negotiation and establishment of the connections, the SEALDD client gets the mapping information between application traffic and SEALDD flow ID. The SEALDD server gets the mapping information between the SEALDD flow ID and the SEALDD-S connection. Upon receiving application traffic from VAL client, the SEALDD maps it to SEALDD traffic with SEALDD traffic descriptors as negotiated with SEALDD server in step 6 and step 7. The SEALDD traffic is sent to the SEALDD server. The SEALDD server maps the SEALDD traffic to the application traffic according to the stored SEALDD traffic descriptor, SEALDD client ID and SEALDD flow ID. The SEALDD server sends the recovered application traffic to the address provided by VAL server in step 1, via the connection established in step 2 or 8 according to the mapping information. The downlink application traffic sent from VAL server to VAL client is processed similarly. If packetization indication indicates that SEALDD layer needs to perform packetization, the SEALDD server performs packetization and sends streaming data (e.g. RTP packet) via SEALDD-UU user plane (e.g. SEALDD/UDP/IP) to the SEALDD client for downlink application traffic. Similarly, the SEALDD client performs packetization and sends streaming data (e.g. RTP packet) via SEALDD-UU user plane (e.g. SEALDD/UDP/IP) to the SEALDD server for uplink application traffic. The SEALDD server and client also perform PDU Set inclusion (e.g. in RTP extension as defined in 3GPP TS 26.522 [TS26522]), and if needed, stream session and transport management (e.g. RTCP, RTSP). The SEALDD server receives any UE location change notification using SEAL LM services defined in 3GPP TS 23.434[4] clause 9.3.12, then the SEALDD server performs the data delivery in alignment with the geofence policy. If the UE is in the forbidden location or not allowed for the given VAL service to send/receive data as per the Geofence policy, then the SEALDD server performs action like releases the connection and informs the VAL server that UE is not reachable because in a forbidden location using connection event status procedure. If UE enters the allowed location area then the SEALDD server initiates the connection establishment using the procedure defined in clause 9.2.2.3. 9.2.2.3 SEALDD enabled regular data transmission connection establishment based on policy The SEALDD servers has Data Delivery (DD) policy being provisioned. Before the application communication between VAL client and VAL server starts, the DD policy is enforced by the SEALDD server to establish the SEALDD connection. Pre-conditions: 1. The SEALDD server has DD policies available. Figure 9.2.2.3-1: Policy enforced by SEALDD server for connectivity 1. The VAL server subscribes to SEALDD event exposure for connection status using the procedure defined in clause 9.2.2.6. 2. When the time for data transmission is about to start, the SEALDD server enforces the policy to trigger regular data transmission connection establishment. If spatial condition for UE is provided, the SEALDD server also ensures the UE’s location requirement is satisfied when establishing regular data transmission connection (e.g. by using NEF service for monitoring UE location or SEAL location service for UE entering area of interest). 3. If there is a special routing requirement for SEALDD user plane traffic (e.g. running on a specific slice and DNN), the SEALDD server interacts with 3GPP CN to provision service specific parameters with NEF as described in 3GPP TS 23.502 [6], clause 4.15.6.10 and clause 4.15.6.7. If there are QoS requirements in the DD policy, the SEALDD server also applies QoS to ensure the quality for SEALDD traffic by utilizing NEF/PCF/NRM/EES service for QoS adjustment. Specifically, the SEALDD server relies on the northbound Policy Authorization Service API exposed by the PCF as specified in 3GPP TS 23.502 [6] and 3GPP TS 23.503 [7], if the SEALDD server is connected to the PCF via the N5 reference point, or the northbound AF Session with QoS Service API and/or the PFD Management northbound APIs exposed by the NEF as specified in 3GPP TS 23.502 [6] and 3GPP TS 23.503 [7], if the SEALDD server is connected to the PCF via NEF. SEALDD may also rely upon the EES Session with QoS API as specified in 3GPP TS 23.558 [10] and/or the NRM QoS functionality as described in 3GPP TS 23.434 [4]. For application multi-modal service, the SEALDD server derives PDU Set related assistance information based on received VAL service ID and/or VAL server ID, and protocol description for interacting with NEF/PCF. If the DD policy specifies failure detection report, the SEALDD server may subscribe to CN analytics (e.g. DN performance analytics) from NEF/NWDAF and further notify data delivery status of application traffic to VAL client (via SEALDD client) and VAL server based on analytics result. 4. The SEALDD server allocates an IP address and port for sending and receiving packet over SEAL-S reference point, then SEALDD server sends SEALDD connection establishment notification to the VAL server with VAL service ID, the IP address and port. 5-6. The SEALDD server allocates an IP address and port for sending and receiving packet over SEAL-Uu reference point, then SEALDD server sends regular data transmission connection establishment request to the SEALDD client with SEALDD flow ID, VAL service ID, the IP address and port. The SEALDD server may send the protocol description (UL related info) received from the VAL server to the SEALDD client in the SEALDD regular transmission connection establishment request. The request is responded by the SEALDD client. UE IP address (and port) may be included by the SEALDD client in the response or sent in a separate update message by SEALDD client if a different UE IP address is to be used in SEALDD connection user plane. NOTE 1: Step 4 and step 5 can be done in parallel. NOTE 2: Step 5 can be sent via PDU session (if exist) or via application triggering (if no PDU session exists). 7. The SEALDD client further notifies the VAL client about the SEALDD connection being established. Upon receiving application traffic from VAL client (not shown in the figure), the SEALDD client sends it to SEALDD server in SEALDD traffic. The SEALDD server identifies application traffic based on the VAL service ID and further sends the application traffic to VAL server. The downlink application traffic sent from VAL server to VAL client is processed similarly. If packetization indication indicates that SEALDD layer needs to perform packetization, the SEALDD server performs packetization and sends streaming data (e.g. RTP packet) via SEALDD-UU user plane (e.g. SEALDD/UDP/IP) to the SEALDD client for downlink application traffic. Similarly, the SEALDD client performs packetization and sends streaming data (e.g. RTP packet) via SEALDD-UU user plane (e.g. SEALDD/UDP/IP) to the SEALDD server for uplink application traffic. The SEALDD server and client also perform PDU Set inclusion (e.g. in RTP extension as defined in 3GPP TS 26.522 [TS26522]), and if needed, stream session and transport management (e.g. RTCP, RTSP). 9.2.3.1 SEALDD enabled regular transmission request Table 9.2.3.1-1 describes the information flow from the VAL server to the SEALDD server for requesting the regular application transmission service. Table 9.2.3.1-1: SEALDD enabled Regular transmission request Information element; Status; Description. VAL server ID; M; Identity of the VAL server. VAL service ID; O; Identity of the VAL service. UE identity; O; Identifier of specific UE, e.g. UE ID. SEALDD-S Data transmission connection information; M; Address/port and/or URL of the VAL server to receive the application packets from the SEALDD server. QoS information; O; QoS information provided by VAL server. VAL server’s total bandwidth limit; O (See NOTE); The total bandwidth limit of VAL server, including UL/DL. VAL users’ bandwidth limit; O (See NOTE); The bandwidth limits (i.e. minimum bandwidth requirement and maximum bandwidth limit) for VAL users, including UL/DL. Protocol description; O; The protocol description of VAL traffic. It includes header extension information (e.g. RTP extension with PDU set), packetization indication, payload type and format (e.g. H.264/RTP, H.265/RTP, H.264, H.265). Header extension information is only applicable when payload indicates RTP.. NOTE: These IEs are used for the SEALDD enabled bandwidth control for different VAL users.. 9.2.3.7 SEALDD connection status subscription request Table 9.2.3.7-1 describes the information flow from the VAL server to SEALDD server to subscribe to SEALDD connection status information. Table 9.2.3.7-1: SEALDD connection status subscription request Information element; Status; Description. VAL server ID; M; Identity of the VAL server. Event ID list; M; Identifies a list of events such as establishment, release, congestion report.. VAL service ID; O; Identity of the VAL service. Identity; O; Identifier of VAL UE or VAL user.. SEALDD-S Data transmission connection information; M; Address/port of the VAL server to send/receive the application packets to/from the SEALDD server.. Immediate reporting flag; O; Indicates the immediate reporting of connection status notification. SEALDD client connection status check periodicity; O; Indicates the frequency to perform SEALDD client connection status check. Protocol description; O; The protocol description of VAL traffic. It includes header extension information (e.g. RTP extension with PDU set), packetization indication, payload type and format (e.g. H.264/RTP, H.265/RTP, H.264, H.265). Header extension information is only applicable when payload indicates RTP.. 9.2.3.3 SEALDD regular transmission connection establishment request Table 9.2.3.3-1 describes the information flow from the SEALDD client to the SEALDD server or from the SEALDD server to the SEALDD client for requesting the regular SEALDD connection establishment. Table 9.2.3.3-1: SEALDD regular transmission connection establishment request Information element; Status; Description. Requestor ID; M; Identity of the requestor (SEALDD client or SEALDD server).. SEALDD flow ID; M (See NOTE 1); Identity of the SEALDD flow.. VAL server ID; O; Identity of the VAL server, applicable for SEALDD client side initiated request.. VAL service ID; O; Identity of the VAL service. Selected VAL server endpoint; M; Endpoint of the selected VAL server. SEALDD traffic descriptor; O; SEALDD traffic descriptor (e.g. address, port, URL, transport layer protocol) of the SEALDD client side (for client side initiated request) or the SEALDD server side (for server side initiated request) used to establish SEALDD connection.. VAL UE/user identity; O; The VAL user ID of the VAL user or VAL UE ID.. SEALDD communication lifetime; O; Identifies the DD communication lifetime, applicable for SEALDD server side initiated request.. Capability for BAT and periodicity adaptation; O (See NOTE 2); Indicates BAT and periodicity adaptation capability for SEALDD client in client side initiated request.. Transmission assistance info; O (See NOTE 2); Indicates transmission assistance information for uplink SEALDD traffic in client side initiated request. It includes BAT, BAT window, periodicity, and periodicity range.. L4S feedback capability; O (See NOTE 3); Identifies the L4S feedback capability (i.e. ECN identification, L4S feedback) for client side initiated request. Protocol description; O; The protocol description of VAL traffic. It includes header extension information (e.g. RTP extension with PDU set), packetization indication, payload type and format (e.g. H.264/RTP, H.265/RTP, H.264, H.265). Header extension information is only applicable when payload indicates RTP.. NOTE 1: The SEALDD flow ID is used by the SEALDD client and SEALDD server to identify different application traffic, and it is mapped to the identifiers of the application traffic and data transmission session. NOTE 2: Only one of these IEs may be present in the message. NOTE 3: This IE is used for the SEALDD enabled congestion control for VAL applications, as specified in clause 9.8.2.2.. 9.2.3.4 SEALDD regular transmission connection establishment response Table 9.2.3.4-1 describes the information flow from the SEALDD server to the SEALDD client or from the SEALDD client to the SEALDD server for responding to the regular SEALDD connection establishment. Table 9.2.3.4-1: SEALDD regular transmission connection establishment response Information element; Status; Description. Result; M; Indicates the success or failure of establishing the SEALDD connection.. SEALDD traffic descriptor; O; SEALDD traffic descriptor (e.g. address, port, URL, transport layer protocol) of the SEALDD server side (for client side initiated request) or the SEALDD client side (for server side initiated request) used to establish SEALDD connection.. Pending timer; O (See NOTE 1); The pending timer to trigger the re-connection from SEALDD client when bandwidth limit check is failed.. Suggested traffic transmission bandwidth; O (See NOTE 1); The suggested traffic transmission bandwidth used by SEALDD client or SEALDD server to perform bandwidth control for VAL users, including UL/DL.. Cause; O (See NOTE 2); Indicates the reason for the failure, e.g. SEALDD policy mismatch.. Capability for BAT and periodicity adaptation; O (See NOTE 3); Indicates BAT and periodicity adaptation capability for SEALDD client in server side initiated request.. Capability for BAT and periodicity adaptation; O (See NOTE 3); Indicates BAT and periodicity adaptation capability for SEALDD client in server side initiated request.. Protocol description; O; The protocol description of VAL traffic. It includes header extension information (e.g. RTP extension with PDU set), packetization indication, payload type and format (e.g. H.264/RTP, H.265/RTP, H.264, H.265). Header extension information is only applicable when payload indicates RTP.. NOTE 1: These IEs are used for the SEALDD enabled bandwidth control for different VAL users, applicable for client side initiated request. NOTE 2: This IE is only present if the Result is failure NOTE 3: Only one of these IEs may be present in the message..

7.7.3	Solution evaluation

This solution addresses KI#8 to support:

-	using SEALDD to facilitate XR traffic between application clients and application servers regarding XR traffic transmission with PDU set inclusion.

7.8	Solution #8: EAS instantiation enhancement to satisfy E2E KPI requirements for XR application

7.8.1	Architecture Impacts

This solution is presented based on the existing EDGEAPP architecture in TS 23.558 [11].

7.8.2	Solution description

7.8.2.1	General

This solution proposes that the enablement layer (i.e. EES) can dertermines the required computing ability requirements of EAS instantiation and/or the required QoS information. By interacting with management system and 5GC, the E2E KPI application requirements of XR application can be guaranteed.

In this solution, the EES gives the initial determinantion about required computing ability and/or the required QoS to satisfy the AC service KPI requirments (i.e. the E2E KPI requirements for XR application). The EES may update the computing ability and/or the QoS information after step 6 for further EAS instantiation, such as according to the real-time monitoring/report about computing quality and transmission quality for the instantiated EAS.

NOTE 1: The E2E KPI requirements mentioned in this paper, indicates the latency requirements of XR application/AC (i.e. the response time in AC service KPI).

7.8.2.2	Procedure

The following figure provides the procedure of EAS instantiation enhancement to satisfy E2E KPI requirement (e.g. low latency) for XR application. The EES determines the required computing ability and/or the required QoS information based on the AC service KPI, the available computing capability (i.e., maximum computing capability) and the available transmission capability for the requested EAS.

Pre-conditions:

- EAS registers to EES according to EAS Registration procedure in clause 8.4.3 of TS 23.558 [11].

- EES registers to ECS according to EES Registration procedure in clause 8.4.4 of TS 23.558 [11].

- The ECSP management system provides the information about instantiable EASs (e.g. instantiable EAS ID, the available computing capability) to the EES.

Figure 7.8.2.2-1: EAS instantiation enhancement procedure

- The EEC performs service provisioning procedure, as described in clause 8.3.3 of TS 23.558 [11].

- The EEC sends the EAS discovery request to the EES, the request includes the AC profile with the required EAS ID, and the AC service KPI (i.e. the E2E KPI requirements for XR application).

- The EAS discovery request triggers the EES to verify whether the instantiation of the EAS indicated in the request is required, as specified in clause 8.12.1 of TS 23.558 [11]. If EAS instantiation is required, the EES can determine the required computing ability and/or the required QoS information based on the AC service KPI, the available computing capability and the available transmission capability for the requested EAS.

NOTE 2: The available computing capability (i.e., maximum computing capability) is provided to EES by the ECSP management system as described in pre-condition, which can be used to as the maximum requested computing ability.

NOTE 3: The EES can obtain the available transmission capability from NWDAF, by utilizing the DN performance analytic in clause 6.14 of TS 23.288 [16], which can be used to as the maximum requested QoS information (i.e. transmission quality).

- If the EES determines that EAS instantiation is needed, the EES can request the ECSP management system to perform the EAS instantiation with the required EAS ID, and the EAS requirements including the required computing ability determined in step 3, as specified in clause 7.1.2.1 of TS 28.538 [17]. Upon reception of the EAS instantiation request, the ECSP management system verifies EAS instantiation requirements and performs EAS instantiation as specified in 3GPP TS 28.538 [17]. After the EAS instantiation is completed, the EES can be informed by ECSP management system with the result of EAS instantiation including the EAS endpoint information.

NOTE 4:  The EES sends a report for a need of the EAS instantiation to the ECSP management system to consider instantiating the requested EAS by invoking an MnS API of the ECSP management system. The ECSP management system has the final decision about EAS instantiation.

Editor's note:	Whether EES can provide required computing ability to ECSP management system is FFS.

- After the EAS instantiation is completed, the EES will receive the EAS profile information via the EAS registration procedure.

- The EES can send the AF request to 5GC via N33/N5 with the application descriptor (i.e. the EAS endpoint information after instantiated by ECSP management system in step 4 or step 5), and the required QoS information dertermined in step 3, by utilizing the AF session with required QoS procedure in clause 4.15.6.6 of TS 23.502 [4].

NOTE 5: In step 6, the EES may update the required QoS information based on receiving the EAS profile about the detailed EAS service KPI, if step 5 happens.

- The EES sends the EAS discovery response to EEC with the EAS endpoint information.

NOTE 6:  If the EAS instatiation is failed in step 4, the step 5 can be skipped, and the EES responds in EAS discovery response with an appropriate failure cause.The XR application client/AC can access the instantiated EAS with the required computing ability and the required QoS information, the E2E KPI application requirements of XR application can be guaranteed.

7.8.3	Solution evaluation

Editor's note:	Solution evaluation is FFS.

7.x	Solution #x: <title>

Editor's Note:	Provide a suitable title for the solution.

7.x.1	Architecture Impacts

Editor's note:	This clause provides the architecture impacts of the solution.

7.x.2	Solution description

Editor's Note:	This clause will describe the solution. Each solution should clearly describe which of the key issues it covers and how.

7.x.3	Solution evaluation

Editor's note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

8	Business Relationships

8.1	General

The business relationships outlined in TS 23.434 and TS 23.433[6] remain applicable, with the following modifications:

-	the XR service provider acting as the VAL service provider;

-	the XRAPP service provider acting as the SEAL/SEALDD service provider.

9	Overall evaluation

Editor's Note:	This clause will provide evaluation of different solutions.

10	Conclusions

10.1	General conclusions

Editor's note:	This clause will provide general conclusions for the study.

10.2	Conclusions of key issue #x

Editor's Note:	This clause will provide conclusions for the specific key issue.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2023-11 | SA6#58 |  |  |  |  | TR skeleton | 0.0.0 |
| 2023-12 | SA6#58 |  |  |  |  | TR Skeleton agreed in SA6#58: S6-234018, S6-234042, S6-234020, S6-234087, S6-234107 Editorial changes by the rapporteur | 0.1.0 |
| 2024-03 | SA6#59 |  |  |  |  | PCR agreed in SA6#59: S6-240150, S6-240679, S6-240683, S6-240708. | 0.2.0 |
| 2024-04 | SA6#60 |  |  |  |  | PCR agreed in SA6#60: S6-241618, S6-241490, S6-241491, S6-241654, S6-241495, S6-241640, S6-241497, S6-241498, S6-241499, S6-241500, S6-241563, S6-241564, S6-241565, S6-241494. Editorial changes by the rapporteur | 0.3.0 |