| 3GPP TR 23.700-53 V0.3.0 (2022-05) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on access traffic steering, switching and splitting support in the 5G system architecture; Phase 3 (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

1	Scope	8

2	References	8

3	Definitions of terms and abbreviations	10

3.1	Terms	10

3.2	Symbols	10

3.2	Abbreviations	10

4	Architectural Assumptions and Requirements	10

5	Key Issues	10

5.1	Key Issue #2: New steering functionalities for non-TCP traffic	11

5.1.1	Description	11

5.2	Key Issue #3: Support of redundant traffic steering	11

5.2.1	Description	11

5.3	Key Issue #5: Switching traffic of an MA PDU Session between two non-3GPP access paths	12

5.3.1	Description	12

5.4	Key Issue #6: Supporting MA PDU Session with one 3GPP access path via 5GC and one non-3GPP access path via ePDG/EPC	13

5.4.1	Description	13

6	Solutions	14

6.0	Mapping of Solutions to Key Issues	14

6.1	Solution #3.1: New steering mode - Redundancy steering mode with packet loss rate	14

6.1.1	Introduction	14

6.1.2	High-level Description	14

6.2	Solution #6.1: Support non-3GPP access leg of MA-PDU Session with PDN connection in EPC	15

6.2.1	Introduction	15

6.2.2	High-level Description	15

6.2.3	Impacts on services, entities, interfaces and IETF protocols	17

6.3	Solution #2.1: MP-DCCP based Steering Functionality	17

6.3.1	Introduction	17

6.3.2	Description	18

6.3.3	MA PDU Session Establishment procedure	22

6.3.4	MA PDU Session Modification procedure	24

6.3.5	Example of MP-DCCP-LL Operation	24

6.3.6	Support of Steering Modes	27

6.3.7	Support of Re-Ordering	27

6.3.8	Impacts on services, entities, interfaces and IETF protocols	28

6.4	Solution #3.2: Redundant traffic steering triggered by AF	29

6.4.1	Introduction	29

6.4.2	High-level Description	29

6.4.3	Procedures	29

6.4.3.0	General	29

6.4.3.1	Procedure of AF providing PLR threshold value via Nnef_TrafficInfluence_Create/Update operation service	30

6.4.3.2	Procedure of AF providing PLR threshold value via Nnef_AFsessionWithQoS_Create/Update operation service	31

6.4.4	Impacts on Existing Nodes and Functionality	31

6.5	Solution #3.3: New traffic duplication steering mode	32

6.5.1	Introduction	32

6.5.2	High-level Description	32

6.5.3	Procedures	34

6.5.4	Impacts on Existing Nodes and Functionality	34

6.6	Solution #3.4: Redundant steering mode with duplication information and trigger mechanisms	35

6.6.1	Introduction	35

6.6.2	High-level Description	35

6.6.2.1	Provision of RSM related parameters to UE and UPF	35

6.6.2.2	Triggering the usage of a MA PDU Session with RSM	35

6.6.3	Procedures	36

6.6.4	Impacts on Existing Nodes and Functionality	37

6.7	Solution #5.1: Support traffic switching between two non-3GPP paths	38

6.7.1	Introduction	38

6.7.2	High-level Description	38

6.7.3	Procedures	38

6.7.4	Impacts on Existing Nodes and Functionality	40

6.8	Solution #5.2: Delaying UDM Registration until non-3GPP access switching completes	41

6.8.1	Introduction	41

6.8.2	High-level Description	41

6.8.3	Procedures	41

6.8.4	Impacts on Existing Nodes and Functionality	43

6.9	Solution #5.3: Path switching between non-3GPP accesses	43

6.9.1	Introduction	43

6.9.2	High-level Description	43

6.9.3	Procedures	44

6.9.4	Impacts on Existing Nodes and Functionality	46

6.10	Solution #5.4: Non-3GPP access path switching in MA PDU Session	47

6.10.1	Introduction	47

6.10.2	High-level Description	47

6.10.3	MA PDU Session Access Path Switching procedure	48

6.10.4	Impacts on Existing Nodes and Functionality	50

6.11	Solution #2.2: MPQUIC steering functionality using UDP proxying over HTTP	50

6.11.1	Introduction	50

6.11.2	High-level Description	51

6.11.3	Procedures	54

6.11.4	Impacts on Existing Nodes and Functionality	59

6.12	Solution #2.3: MPQUIC steering functionality using IP proxying over HTTP	59

6.12.1	Introduction	59

6.12.2	High-level Description	59

6.12.3	Procedures	62

6.12.4	Impacts on Existing Nodes and Functionality	66

6.13	Solution #2.4: Limiting MA-PDU Per-Packet Overhead	67

6.13.1	Introduction	67

6.13.2	High Level Description	67

6.13.3	Procedures	67

6.13.4	Impacts on Existing Nodes and Functionality	68

6.14	Solution #3.5: Redundant Traffic Steering Mode Activation/Deactivation	68

6.14.1	Introduction	68

6.14.2	High Level Description	68

6.14.3	Procedures	70

6.14.4	Impacts on Existing Nodes and Functionality	71

6.15	Solution #3.6: Redundant steering mode	71

6.15.1	Introduction	71

6.15.2	High-level Description	71

6.15.3	Procedures	72

6.15.4	Impacts on Existing Nodes and Functionality	72

6.16	Solution #5.5: Non-3GPP path switch during Registration in new non-3GPP access	72

6.16.1	Introduction	72

6.16.2	High-level Description	72

6.16.3	Procedures	72

6.16.4	Impacts on Existing Nodes and Functionality	74

6.17	Solution #5.6: Consolidated solution for traffic switching between two non-3GPP access paths	75

6.17.1	Introduction	75

6.17.2	High-level Description	75

6.17.3	Procedures	75

6.17.4	Impacts on Existing Nodes and Functionality	77

6.X	Solution #X: <Solution Title>	78

6.X.1	Introduction	78

6.X.2	High-level Description	78

6.X.2	Procedures	78

6.X.3	Impacts on Existing Nodes and Functionality	78

7	Evaluation	79

7.1	Evaluation for KI #6: Support non-3GPP access leg of MA-PDU Session with PDN connection in EPC	79

8	Conclusions	79

Annex A: Change history	80

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, certain modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

NOTE 1:	The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

NOTE 2:	The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

NOTE 3:	The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

NOTE 4:	The constructions "can" and "cannot" shall not to be used as substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

NOTE 5:	The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The purpose of this Technical Report is to enhance the ATSSS feature by investigating solutions that can support the following capabilities:

1)	Support new steering functionalities that can steer/switch/split non-TCP traffic flows (e.g. UDP traffic flows and IP traffic flows). Two types of such steering functionalities are studied: (i) a steering functionality based on the QUIC protocol and its multipath extensions, and (ii) a steering functionality based on the DCCP protocol and its multipath extensions.

2)	Support redundant traffic steering for both GBR and non-GBR traffic flows. With redundant traffic steering, a traffic flow (GBR or non-GBR) can be replicated on multiple access paths and, therefore, can improve transmission reliability and reduce packet latency.

3)	Support traffic switching between one non-3GPP access path, from a UE to a N3IWF in a PLMN, and another non-3GPP access path, from the UE to a TNGF in the same PLMN.

4)	Support the establishment of a MA PDU Session with one 3GPP access path via 5GC and one non-3GPP access path via ePDG/EPC. This is to complement the existing ATSSS capability that supports the establishment of a MA PDU Session with one non-3GPP access path via 5GC and one 3GPP access path via EPC.

For each of the above capabilities, the conclusions of the study identify whether a solution is required for the normative phase and, if required, which solution to consider in the normative phase.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TR 23.700-93: "Study on access traffic steering, switch and splitting support in the 5G System (5GS) architecture; Phase 2".

[6]	IETF RFC 9000: "QUIC: A UDP-Based Multiplexed and Secure Transport".

[7]	IETF RFC 9001: "Using TLS to Secure QUIC".

[8]	IETF RFC 9002: "QUIC Loss Detection and Congestion Control".

[9]	IETF RFC 9221: "An Unreliable Datagram Extension to QUIC".

[10]	draft-ietf-quic-multipath: "Multipath Extension for QUIC".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[11]	IETF RFC 4340: "Datagram Congestion Control Protocol (DCCP)".

[12]	draft-ietf-tsvwg-multipath-dccp: "DCCP Extensions for Multipath Operation with Multiple Addresses".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[13]	3GPP TS 24.193: "Access Traffic Steering, Switching and Splitting (ATSSS); Stage 3".

[14]	3GPP TS 24.302: "Access to the 3GPP Evolved Packet Core (EPC) via non-3GPP access networks; Stage 3".

[15]	IETF RFC 4336: " Problem Statement for the Datagram Congestion Control Protocol (DCCP)".

[16]	draft-amend-tsvwg-multipath-framework: "A multipath framework for UDP traffic over heterogeneous access networks".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[17]	IEEE AINAW, "Out-of-Order Transmission for In-Order Arrival Scheduling for Multipath TCP", DOI:10.1109/WAINA.2014.122.

[18]	draft-amend-iccrg-multipath-reordering: "Multipath sequence maintenance".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[19]	IETF RFC 4341: "Profile for Datagram Congestion Control Protocol (DCCP) Congestion Control ID 2: TCP-like Congestion Control".

[20]	IETF RFC 4342: "Profile for Datagram Congestion Control Protocol (DCCP) Congestion Control ID 3: TCP-Friendly Rate Control (TFRC)".

[21]	IETF RFC 5622: "Profile for Datagram Congestion Control Protocol (DCCP) Congestion ID 4: TCP-Friendly Rate Control for Small Packets (TFRC-SP)".

[22]	draft-romo-iccrg-ccid5: "Profile for Datagram Congestion Control Protocol (DCCP) Congestion Control ID 5".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[23]	3GPP TS 29.512: "5G System; Session Management Policy Control Service; Stage 3".

[24]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane nodes".

[25]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[26]	3GPP TR 23.793: "Study on access traffic steering, switch and splitting support in the 5G System (5GS) architecture".

[27]	draft-ietf-masque-connect-udp: "UDP Proxying Support for HTTP".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[28]	draft-ietf-quic-http: "Hypertext Transfer Protocol Version 3 (HTTP/3)".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[29]	draft-ietf-masque-h3-datagram: "Using Datagrams with HTTP".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[30]	draft-ietf-httpbis-h3-websockets: "Bootstrapping WebSockets with HTTP/3".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[31]	draft-ietf-masque-connect-ip: "IP Proxying Support for HTTP".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[32]	IETF RFC 5225: "RObust Header Compression Version 2 (ROHCv2): Profiles for RTP, UDP, IP, ESP and UDP-Lite".

[33]	IETF RFC 6846: "RObust Header Compression (ROHC): A Profile for TCP/IP (ROHC-TCP)".

[34]	3GPP TS 38.323: "Packet Data Convergence Protocol (PDCP) specification".

# 3	Definitions of terms and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1], in TS 23.501 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1] or in TS 23.501 [2].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], in TS 23.501 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] or in TS 23.501 [2].

# 4	Architectural Assumptions and Requirements

The study in this technical report is based on the following architectural requirements and assumptions:

-	The study takes the Rel-17 ATSSS architecture (see TS 23.501 [2]) as a baseline.

-	It is assumed that an MA PDU Session is required for supporting all ATSSS capabilities. Only ATSSS enhancements using an MA PDU Session are considered.

-	The study may impact the core network (5GC and/or EPC) but it shall not impact the access network (e.g. NG-RAN, W-5GAN, etc.).

-	All ATSSS steering functionalities (existing and new) shall reside in the UE and in an UPF. Steering functionalities outside either of the UE or the UPF are not considered.

-	Any new steering functionality shall be based on the multipath extensions of the QUIC or DCCP protocols as defined in IETF.

-	New ATSSS capabilities shall either be able to co-exist with the existing ATSSS capabilities or any dependencies between new and existing capabilities shall be explicitly defined.

-	The study considers, not only ATSSS-capable UEs, but also ATSSS-capable 5G-RGs. The conclusions of the study identify which aspects / solutions are applicable only to ATSSS-capable UEs and which are applicable only to ATSSS-capable 5G-RGs.

NOTE:	Co-ordination with BBF and CableLabs will take place as needed during the study.

# 5	Key Issues

NOTE:	The numbering of the key issues below corresponds to the numbering of the Work Tasks in the approved ATSSS_ph3 study item (see SP-211612).

## 5.1	Key Issue #2: New steering functionalities for non-TCP traffic

### 5.1.1	Description

This key issue aims at studying new steering functionalities (in addition to the existing ATSSS-LL and MPTCP steering functionalities defined in TS 23.501 [2]), which can be used to support steering, switching and splitting of non-TCP traffic flows (e.g. UDP traffic flows and IP traffic flows). Presently, traffic splitting of non-TCP traffic flows is not fully supported with the ATSSS-LL because this steering functionality may introduce out of order delivery, which can severely impact the transport performance.

Editor's note:	Whether support of steering, switching and splitting of Ethernet traffic flows is required is FFS.

More specifically, this key issue aims to:

1)	Continue the Rel-17 study of the QUIC-based steering functionality and its multipath extensions by considering some of the aspects that were left open (see clause 8.2 of TR 23.700-93 [5]). The resolution of these aspects may lead to new solutions, in addition to those specified in TR 23.700-93 [5], and any of them should support per-packet splitting. For example, it will be considered whether the QUIC-based steering functionality will apply other IETF protocols, such as the MASQUE protocol, and whether a single multipath QUIC connection can support one or multiple steering modes.

The study of the QUIC-based steering functionality is based on the QUIC protocol [6], its multipath extensions (e.g. draft-ietf-quic-multipath [10]) and, possibly, on other relevant documents specified by IETF, such as RFC 9001 [7], RFC 9002 [8], draft-ietf-quic-datagram [9].

Any security aspects associated with the QUIC protocol mandating the usage of TLS 1.3 for key exchange, authentication, and negotiation of security and performance parameters (see RFC 9001 [7]), will be studied in conjunction with SA WG3.

2)	Study a new steering functionality based on the DCCP protocol RFC 4340 [11] and its multipath extensions draft-ietf-tsvwg-multipath-dccp [12] that provide support for per-packet splitting.

The conclusions of the study will identify which one of the above two steering functionalities may be specified in the normative phase.

This key issue shall also consider the following additional aspects:

-	How the new steering functionalities can co-exist with MPTCP and ATSSS-LL;

-	What is the impact on the user plane performance (e.g. additional overhead) for each one of the new steering functionalities;

-	Whether it is needed and how to negotiate the support of the new steering functionalities between the UE and the network;

-	Whether it is needed and how to enhance PCC rules, ATSSS rules and N4 rules to support the new steering functionalities;

-	UE impacts in order to support each one of the new steering functionalities;

-	How to treat out-of-order delivery caused by per packet-splitting.

## 5.2	Key Issue #3: Support of redundant traffic steering

### 5.2.1	Description

This key issue is to enable the transmitter (UE for UL traffic and UPF for DL traffic) to support redundant traffic steering with duplication criteria, in order to improve reliability and latency.

The key issue also studies potential duplication criteria, i.e. how to activate or deactivate packet transmission redundancy on both accesses with additional mechanisms. The duplication criteria are known to UE and UPF, e.g. allowing to decide which DL and/or UL packets need to be duplicated on all access paths of a MA PDU session and which ones not.

This key issue aims to study the following issues:

-	How to support redundant traffic steering mode for different steering functionalities.

NOTE:	The redundant traffic steering mode does not apply to the ATSSS-LL steering functionality.

-	Whether new measurements are needed to support redundant traffic steering mode.

-	Whether and which criteria for traffic duplication should be introduced in order to allow the UE and UPF to decide, e.g. which packets must be duplicated on all access paths of a MA PDU session and which packets must be send over only one access path.

-	If duplication criteria are introduced, how ATSSS and MA rules should be modified to allow to indicate the duplication criteria to UE and UPF.

-	How the UE and UPF operate based on the modified ATSSS and MA rules for both GBR and non-GBR traffic. This includes the definition of UE and UPF behaviours for the case in which traffic is not duplicated on both access paths as it does not fulfil the configured duplication criteria.

-	Whether and how to introduce mechanisms to activate and deactivate packet duplication in a dynamic manner, e.g. triggered by network entities or application functions.

-	How the receiver (UPF for UL traffic and UE for DL traffic) will treat duplicated packets.

## 5.3	Key Issue #5: Switching traffic of an MA PDU Session between two non-3GPP access paths

### 5.3.1	Description

This key issue aims to study how the data traffic of an MA PDU Session can be switched between one non-3GPP access path, from the UE to a N3IWFin a PLMN, and another non-3GPP access path, from the UE to a TNGF in the same PLMN.

The following topics are considered:

-	How the UE can register to 5GC in order to enable switching the traffic from one non-3GPP access path to another non-3GPP access path.

-	How to switch the traffic of an MA PDU Session between one non-3GPP access path from the UE to a N3IWF and another non-3GPP access path from the UE to a TNGF in the same PLMN. This includes how the UE and the network take decision to perform the switch of the traffic.

-	Study how existing steering modes and steering functionalities, as well as new steering modes and steering functionalities (defined in this TR) can be reused, or, if needed, be modified to allow switching the traffic from one non-3GPP access path to another non-3GPP access path.

The study of the above topics will be based on the following assumptions:

-	Both non-3GPP access paths traverse the same PLMN.

-	One non-3GPP access path is using an N3IWF, while the other non-3GPP access path is using a TNGF.

-	After switching the traffic, only one UE registration via non-3GPP access may exist.

-	If the UE is able to access the same PLMN directly using 3GPP radio technology, then the UE may have an MA PDU Session with three access paths (two non-3GPP and one 3GPP) for the duration of switching the traffic from a source non-3GPP access path to a target non-3GPP access path.

-	Impact to the UE and network should be minimized.

NOTE 1:	In conclusion if time permits, it can be considered whether the selected solution enables also the traffic switching between one non-3GPP access path (e.g. from the UE to a N3IWF), and another non-3GPP access path (e.g. from the UE to a TNGF) per single access PDU session.

NOTE 2:  During the conclusion phase, it will be considered whether traffic switching between two non-3GPP access paths using same RAT-type can be also supported by the solution selected for the normative phase.

## 5.4	Key Issue #6: Supporting MA PDU Session with one 3GPP access path via 5GC and one non-3GPP access path via ePDG/EPC

### 5.4.1	Description

This key issue considers how to support the establishment of a MA PDU Session with one 3GPP access path via 5GC and one non-3GPP access path via ePDG/EPC. A solution for this key issue complements the existing ATSSS solution that supports the establishment of a MA PDU Session with one non-3GPP access path via 5GC and one 3GPP access path via EPC (see clause 4.22.2.3 of TS 23.501 [2]).

The key issue addresses the following aspects:

-	How to establish a MA PDU session with a non-3GPP access path over ePDG/EPC and a 3GPP access path over 5GC.

-	How to establish an PDN Connection in EPC using the S2b procedures with untrusted non-3GPP access and designate this PDN Connection as user-plane resource associated with a MA PDU Session.

-	How to transfer a non-3GPP access path over ePDG/EPC (while keeping the 3GPP access path over 5GC) to a trusted non-3GPP access path over 5GC.

NOTE:	Handover between non-3GPP of EPC and non-3GPP access of 5GC will not be defined.

-	The impact on UE, ePDG, EPC and 5GC.

-	Which steering functionalities and which steering modes can be applied to an MA PDU Session with a non-3GPP access path over ePDG/EPC and a 3GPP access path over 5GC.

It is assumed that:

-	The UE can attach to EPC via an ePDG using the S2b procedures (see clause 7.2 of TS 23.402 [25]) and can simultaneously register with 5GC over 3GPP access.

-	The study is restricted to EPC attach using the S2b procedures via untrusted non-3GPP access. EPC attach using S2c procedures via untrusted non-3GPP access are not considered.

-	Solutions are preferred that re-use (as much as possible) the existing concepts and procedures for enabling an MA PDU Session with one non-3GPP access path via 5GC and one 3GPP access path via EPC, as defined in clause 4.22.2.3 of TS 23.501 [2].

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solution # | Solution Title | Key Issue(s) |
| --- | --- | --- |
| #2.1 | MP-DCCP based Steering Functionality | #2 (DCCP) |
| #2.2 | MPQUIC steering functionality using UDP proxying over HTTP | #2 (QUIC) |
| #2.3 | MPQUIC steering functionality using IP proxying over HTTP | #2 (QUIC) |
| #2.4 | Limiting MA-PDU Per-Packet Overhead | #2 |
| #3.1 | New steering mode - Redundancy steering mode with packet loss rate | #3 |
| #3.2 | Redundant traffic steering triggered by AF | #3 |
| #3.3 | New traffic duplication operation | #3 |
| #3.4 | Redundant steering mode with duplication information | #3 |
| #3.5 | Redundant Traffic Steering Mode Activation/Deactivation | #3 |
| #3.6 | Redundant steering mode | #3 |
| #5.1 | Support traffic switching between two non-3GPP paths | #5 |
| #5.2 | Delaying UDM Registration until non-3GPP access switching completes | #5 |
| #5.3 | Path switching between non-3GPP accesses | #5 |
| #5.4 | Non-3GPP access path switching in MA PDU Session | #5 |
| #5.5 | Non-3GPP path switch during Registration in new non-3GPP access | #5 |
| #5.6 | Consolidated solution for traffic switching between two non-3GPP access paths | #5 |
| #6.1 | Support non-3GPP access leg of MA-PDU Session with PDN connection in EPC | #6 |

## 6.1	Solution #3.1: New steering mode - Redundancy steering mode with packet loss rate

### 6.1.1	Introduction

This solution addresses KI#3 on support of redundant traffic steering.

### 6.1.2	High-level Description

The redundancy steering mode was discussed during both Rel-16 and Rel-17 ATSSS study phase, as described in clause 6.3.1.1, 6.4.1 of TR 23.793 [26] for the loss rate sensitive traffic, such as IMS singling, video, and some TCP-based traffic, and the solution 4 in clause 6.4 of TR 23.700-93 [5]. It allows the traffic transmitted via 3GPP and non-3GPP accesses in a redundant way to achieve the lowest latency and to lower the loss rate.

It is proposed to further enhance the redundancy steering mode as described in TR 23.793 [26], with which the traffic will always be transmitted over both accesses once applied, to make it possible that the traffic transmission goes via both accesses, if necessary, or via only one access to save the transport resource. In details, when the traffic is allowed on both accesses and the packet loss rate (PLR) is provided with the redundancy steering mode:

-	The UE and the UPF can select any one of the accesses (based on implementation) to transport the traffic, if both accesses can satisfy the PLR requirement.

-	If only one access can satisfy the PLR value, this access is applied to transport the traffic.

-	Otherwise, redundant transmission over both accesses is triggered.

For example, if the loss rate on one access does not exceed the PLR threshold, then only this one access is applied, otherwise, redundant transmission is triggered.

When the traffic is allowed on both accesses and the redundancy steering mode is provided without a PLR threshold value, both accesses are always applied for the redundant transmission.

See below Figure 6.1.2-1 for details, where UL packet flow is taken as an example. The DL packet flow shares the same mechanism.

This redundancy steering mode can be applied by the MPTCP functionality with new requirements to be derived from this study/solution.

NOTE 1:	The redundant transmission algorithm required by the redundancy steering mode with packet loss rate relies on the implementation of the MPTCP by the OS today.

NOTE 2:	The redundancy steering mode with packet loss rate does not apply to the ATSSS-LL steering functionality.

Editor's note:	How the receiver will treat duplicated packets in case of MP-QUIC or MP-DCCP is FFS.

Editor's note:	The redundant traffic steering mode for the GBR traffic is FFS.

Editor's note:	Whether threshold values other than packet loss rate, such as RTT, are applicable, or whether and how multiple threshold values can be used together, is FFS.

NOTE:	For the MA PDU session, when the UE performs traffic switching between the 3GPP and non-3GPP accesses in 5GS and the MPTCP functionality is applied for the traffic, the UE and the UPF may trigger the redundant transmission via the 3GPP and non-3GPP access. This redundant transmission can be applied to all the MPTCP traffic with any steering mode.

Editor's note:	How the UE and the UPF decide to trigger the redundant transmission via the 3GPP and non-3GPP access during the traffic switching is FFS.

Figure 6.1.2-1: Redundancy steering mode

## 6.2	Solution #6.1: Support non-3GPP access leg of MA-PDU Session with PDN connection in EPC

### 6.2.1	Introduction

This solution enables a UE and the network to create an MA PDU Session with the non-3GPP access leg of a normal PDU connection in EPC, while keeping the 3GPP access leg in 5GC.

### 6.2.2	High-level Description

In this solution, it is assumed that the UE is able to attach to the EPC via GTP-based S2b interface and simultaneously to register with the 5GC over 3GPP access. The architecture is shown in Figure 6.2.2-1.

Figure 6.2.2-1: Multi-access between EPC and 5GC

The solution as defined in clause 4.22.2.3.2 of TS 23.502 [3] is reused, with the major difference:

-	Extend the ATSSS container and apply APCO IE to transport the ATSSS related parameters.

The key points of the solution are as follows:

When the UE firstly requests a new PDN connection to EPC via non-3GPP access and wants to use this PDN connection as user-plane resource associated with a MA PDU Session:

-	The UE requests establishment of a new PDN connection when the UE is registered via non-3GPP access in EPS using the IPsec tunnel establishment procedure. The UE provides the ATSSS container to ePDG via IKE_AUTH request message. The ATSSS container is extended to include the ATSSS request PCO parameter as defined in clause 6.1.6.2 of TS 24.193 [13]. The ePDG forwards the ATSSS request PCO parameter in APCO IE to the PGW-C/SMF.

-	PGW-C+SMF replies the Create Session Response message to ePDG including APCO. The APCO includes the ATSSS response with the length of two octets PCO parameter as defined in clause 6.1.6.3 of TS 24.193 [13]. The ePDG obtains the ATSSS response parameter from APCO and forwards this information via ATSSS container to the UE via IKE_AUTH response message.

When the UE firstly requests a new MA PDU Session in 5GC/3GPP access and wants to add the user-plane resources on non-3GPP access over EPC, it applies the procedures as defined in clause 4.22.2.3.2 of TS 23.502 [3] and clause 7.2.2.1 of TS 24.302 [14] for the transferring an existing PDU session from 5GS:

-	The UE sends ATSSS container with the ATSSS request PCO parameter in the IKE_AUTH request message to the ePDG including a "handover" indication, an "MA PDU Request" indication and the PDU Session ID of the existing MA PDU Session on non-3GPP access over 5GC. The ePDG forwards the ATSSS request PCO parameter in the APCO to the PGW-C/SMF via Create Session Request message.

-	PGW-C+SMF replies the Create Session Response message to ePDG including APCO with ATSSS response with the length of two octets PCO parameter. The ePDG obtains the ATSSS response parameter from APCO and forwards this information via ATSSS container to the UE via IKE_AUTH response message.

For the 3GPP access, the mobility procedure from 5GS to EPS is to be performed as follows:

-	If the MA PDU Session is transferred to EPS in case of interworking support with N26, or if the UE sends a PDN Connectivity Request with "handover" indication to transfer the MA PDU Session to EPS in case of interworking without N26, the SMF+PGW-C triggers release of the non-3GPP access or 3GPP access in EPC due to no support of MA PDU Session in EPC only, or the UE can initiate the UE Requested PDU Session Release procedure in 5GS for the MA PDU Session.

-	Furthermore, in case of UE operating in dual registration, the UE may decide to keep the MA PDU Session in 5GS, in addition to the above behaviour.

### 6.2.3	Impacts on services, entities, interfaces and IETF protocols

ePDG:

-	Shall be able to support the ATSSS container and apply APCO IE to transport the ATSSS related parameters.

PGW-C+SMF:

-	Shall be able to apply APCO IE to transport the ATSSS related parameters.

## 6.3	Solution #2.1: MP-DCCP based Steering Functionality

### 6.3.1	Introduction

This solution addresses Key Issue #2 "New steering functionalities for non-TCP traffic" proposing MP-DCCP-LL to enable Access Traffic Steering, Switching and Splitting for any type of traffic, e.g. UDP, etc.

The ATSSS feature is to enable a UE the advantage that application traffic can use both access resources to raise the bandwidth or/and access reliability by simultaneously connecting over both 3GPP and non-3GPP accesses. Since release-16, the splitting mode is only available to TCP streams using the ATSSS-HL functionality based on MPTCP. For UDP and even plain IP traffic, it can only be supported by the ATSSS-LL functionality, thus an UDP or even an IP packet flow cannot be efficiently split over both accesses due to missing re-ordering support and to the lack of access path bandwidth capabilities, so actually the increase of bandwidth requirement is not achieved for those types of traffic. This solution is proposed to introduce a new steering method based on MP-DCCP to resolve this issue.

The Datagram Congestion Control Protocol (DCCP) was specified in IETF in RFC 4340 [11] for the transport layer to complement TCP and UDP for traffic that does not require reliable transport and re-transmission (in contrast to TCP traffic), but needs session and congestion control (in contrast to UDP traffic). For an introduction to DCCP please refer to RFC 4336 [15]. DCCP RFC 4340 [11] and its multipath pendant MP-DCCP, draft-ietf-tsvwg-multipath-dccp [12] provide protocol inherent measurement of path characteristics like latency, packet loss and available bandwidth. In that regard MP-DCCP is similar to MPTCP and therefore can support the same steering modes. Different to MPTCP though, is the handling of missing packets. Re-transmission and resulting head-of-line blocking are not under the responsibility of (MP-)DCCP, which is therefore similar to the ATSSS-LL characteristics provided for any type of traffic. This makes it suitable to be applied to Ethernet, IP and higher layer traffic when it becomes integrated with an encapsulation framework, draft-amend-tsvwg-multipath-framework [16], which tunnels these types of traffic over a MP-DCCP connection. Moreover MP-DCCP provides measures to facilitate the process of re-ordering at the termination points of ATSSS. For this purpose, timing and sequencing information are transmitted along with the encapsulated data to facilitate re-ordering or partial reordering of the data received from the 3GPP and non-3GPP access. Both access types provide different path characteristics that let the original data stream scramble during transport after it has been split. Re-ordering helps in the receiving ATSSS entity (UPF or UE) to reconstruct a certain degree order.

For example, MP-DCCP supports similar functionality as provided by ATSSS-LL and ATSSS-HL (MPTCP) for all ATSSS steering modes, for any type of traffic, and provides re-ordering capabilities. MP-DCCP does not introduce encryption but relies on the security layer of 3GPP.

MP-DCCP implementation already supports the following steering modes:

-	Cheapest Pipe First (similar to Priority-based of ATSSS): MP-DCCP priority-based scheduling mechanism allows the operator to assign a cost value to each network path. In case of congestion the next available path is chosen, in case of equal cost paths the path with minimum delay is selected.

-	Handover mode (similar to Active Standby of ATSSS): Data on the prioritized path (main path) are switched to next available one, as soon as prioritized DCCP tunnel becomes unavailable.

-	MinRTT (similar to Smallest Delay of ATSSS): Prioritizes the path with the smallest round trip time.

-	Round-Robin: Alternates PDU distribution between available accesses paths.

-	Fixed-Ratio (similar to Load Balancing of ATSSS): allows distribution of traffic based on a given ratio (e.g. 60:40).

-	Redundancy mode: Data are sent via all available paths to enhance reliability.

-	OTIAS mode: The "Out-of-Order Transmission for In-Order Arrival Scheduling" for multipath transport protocols supports reordering at the receiver side and is defined in IEEE AINAW [17].

And the following re-ordering modes:

-	Passive: Forward PDUs as received (effectively no re-ordering)

-	Static: Out-of order PDUs are hold in a re-ordering buffer until a static timer expires (e.g. 50ms). During this time delayed and missing packets are sorted into the sequence gaps.

-	Adaptive: Like the Static re-ordering mode but with an adaptive timer, which covers the latency difference across the access paths. The adaptive timer value is built from the MP-DCCP timing information sent along with the PDUs.

-	Static + Fast packet loss detection: Like the Static re-ordering mode, but additionally exploit the connection and path sequencing used in MP-DCCP for early packet loss detection, see clause 5.6 of draft-amend-iccrg-multipath-reordering [18]

-	Adaptive + Fast packet loss detection: Like the Adaptive re-ordering mode, but additionally exploit the connection and path sequencing used in MP-DCCP for early packet loss detection, see clause 5.6 of draft-amend-iccrg-multipath-reordering [18].

This solution can be applied to any type of traffic, at least to the UDP based application traffic, with IP-based MA PDU Sessions for various IP versions (IPv4, IPv6, IPv4v6).

### 6.3.2	Description

This solution is largely based on Solution #6 "MPQUIC-LL Steering Functionality" defined in TR 23.700-93 [5] with the main difference of using MP-DCCP instead of MP-QUIC.

The MP-DCCP-LL solution is applicable to applications relying on any type of traffic (plain IP, Ethernet, TCP, UDP), but at least provides ATSSS functionalities to non-TCP based traffic (e.g. UDP, QUIC). MP-DCCP-LL is able to co-exist with ATSSS-LL and MPTCP, even if it can take over their functionality.

To support ATSSS using the MP-DCCP-LL steering functionality, the architecture reference model for ATSSS, as specified in TS 23.501 [2] clause 4.2.10, is enhanced as shown in Figure 6.3.2-1. The MP-DCCP-LL steering functionality is implemented in the UE and in the PSA UPF. The PMF functionality is not needed for an MA PDU Session that applies MP-DCCP-LL.

Figure 6.3.2-1: Reference Architecture for ATSSS using MP-DCCP

The MP-DCCP protocol is specified in draft-ietf-tsvwg-multipath-dccp [12]: "DCCP Extensions for Multipath Operation with Multiple Addresses". It is designed to support the multipath scenario, and provides Steering, Switching, Splitting decisions and optimizations in terms of features and modules for multipath scheduling in order to support, e.g. steering modes as specified for ATSSS at ATSSS sender and re-ordering functionality at ATSSS receiver side. Depending on the traffic direction, the UE or PSA UPF acts as ATSSS sender or ATSSS receiver, respectively. The UE, according to draft-amend-tsvwg-multipath-framework [16], uses the MP-DCCP-LL functionality to establish multiple sub-flows as DCCP tunnels which are set up and operated towards the PSA UPF for bi-directional usage. Within each sub-flow inherent sequencing is given and DCCP congestion control provides real-time path characteristic information, which is required for splitting based steering modes (e.g. priority-based, smallest delay, load-balancing) and packet reordering measures.

The MP-DCCP-LL provides an unreliable tunnelling service between the UE and the UPF that is based on:

1.	The DCCP protocol specified in RFC4340 [11] along with accompanying documents that describe congestion control (CCID2, RFC 4341 [19], CCID3, RFC 4342 [20], CCID4, RFC 5622 [21], CCID5, draft-romo-iccrg-ccid5 [22]); and

2.	The DCCP extension specified in draft-ietf-tsvwg-multipath-dccp [12] for supporting multipath DCCP.

The Figure 6.3.2-2 shows the model of an MA PDU Session that operates using the MP-DCCP-LL steering functionality.

Figure 6.3.2-2: Model of MA PDU Session using MP-DCCP-LL

The MP-DCCP-LL is composed of the following components which can also be found in Figure 6.3.2-3:

-	MP-DCCP Tunnel Client (MDTC): The MDTC operates in the UE as a MP-DCCP client application and provides the following functionality:

-	It establishes N MP-DCCP connections with the MP-DCCP tunnel server (MDTS) in the UPF. The number N of MP-DCCP connections, as well as the destination IP address & port for each MP-DCCP connection, is determined from information contained in the PDU Session Establishment Accept message (see the "MP-DCCP Connection Setup Information" below). Each MP-DCCP connection is established immediately after the setup of the MA PDU Session.

NOTE 1: As explained below, each MP-DCCP connection carries the traffic of one QoS flow only. By using a separate MP-DCCP connection for each QoS flow, we ensure that PDUs belonging to different QoS flows, which may require a different QoS treatment, are not multiplexed in the same MP-DCCP connection, hence, each MP-DCCP connection does not need to support QoS-aware scheduling.

-	It receives UL PDUs from the upper layer and, for each UL PDU:

-	First, it selects a QoS flow based on the QoS rules in the UE.

-	Then, it selects a steering mode to be applied for the UL PDU based on the ATSSS rules in the UE.

-	It compresses the inner headers (e.g. IP, UDP, TCP, Ethernet) of the UL PDU.

Editor's note: It is FFS how the header compression parameters are negotiated between the UE and UPF. It should be investigated whether such negotiation can be supported with the solution #2.4.

-	Finally, it selects a MP-DCCP connection to transmit the PDU based on the selected QoS flow and the selected steering mode. For each established MP-DCCP connection, the MDTC maintains an associated steering mode and QoS flow indicator.

NOTE 2:	It is assumed that the scheduler of a MP-DCCP connection can apply only one steering mode. Therefore, if different steering modes are needed for the traffic of a QoS flow, then different MP-DCCP connections are setup for this QoS flow, one per steering mode.

-	MP-DCCP Tunnel Server (MDTS): The MDTS operates in the UPF as a MP-DCCP server application and provides the following functionality:

-	It accepts the MP-DCCP connections requested by MDTC in the UE.

-	It receives DL PDUs from the upper layer and, for each DL PDU, it selects (a) a QoS flow, (b) a MP-DCCP connection to transmit the PDU, and (c) a steering mode. The MP-DCCP connection is selected based on the N4 (e.g. MAR) rules.

-	It compresses the inner headers (e.g. IP, UDP, TCP, Ethernet) of the DL PDU.

Editor's note: It is FFS how the header compression parameters are negotiated between the UE and UPF. It should be investigated whether such negotiation can be supported with the solution #2.4.

-	MP-DCCP protocol: The DCCP protocol as defined by IETF with the applicable multipath extensions, draft-ietf-tsvwg-multipath-dccp [12]. It receives PDUs from the MDTC in the UE (or the MDTS in the UPF) indicating the MP-DCCP connection to be sent on and, for each PDU, it selects an access type (i.e. a MP-DCCP sub-flow) to transmit the PDU. The access type selection is based on the steering mode associated with the MP-DCCP connection and the measurements provided by the MP-DCCP protocol (e.g. RTT, loss rate, congestion).

The MP-DCCP-LL functionality tunnels PDUs (e.g. IP packets or Ethernet frames) over a multipath DCCP transport. The protocol stack in the UE and UPF is shown in Figure 6.3.2-3. The different IP addresses shown in this figure are explained in the next clause.

Figure 6.3.2-3: Protocol stack for MP-DCCP-LL

In summary, the MP-DCCP-LL steering functionality:

a.	Supports a multipath, unreliable, and transparent bi-directional tunneling service between the UE and UPF.

b.	Does not support retransmission of lost DCCP datagrams but supports loss detection, according to RFC 4340 [11].

c.	Provides mechanisms for reordering of out-of-order arrived DCCP datagrams as described in clause 6.3.7.

d.	Supports congestion control per DCCP sub-flow, based on standardized CC algorithms RFC 4341 [19], RFC 4342 [20], RFC 5622 [21], draft-romo-iccrg-ccid5 [22] or newly developed ones (see, e.g. draft-romo-iccrg-ccid5 [22]). As a result, the UE and the UPF may stop sending datagram frames on a DCCP sub-flow when congestion is detected by the MP-DCCP protocol on this sub-flow.

e.	Supports round-trip, packet loss measurements per DCCP sub-flow, as part of the CCID framework as specified in RFC 4340 [11] and provides available bandwidth estimation.

f.	Supports steering modes compatible with the ATSSS rules, the QoS rules and the N4 rules as defined in Rel-17. One steering mode is supported per MP-DCCP connection.

g.	Support compression of the inner headers (e.g. IP, UDP, TCP, Ethernet) of the UL and DL PDUs.

### 6.3.3	MA PDU Session Establishment procedure

Figure 6.3.3-1: MA PDU Session establishment using MP-DCCP

The above Figure 6.3.3-1 shows how an MA PDU Session is established when (a) the UE indicates support for MP-DCCP-LL and (b) the network accepts to apply MP-DCCP-LL for one or more traffic flows. All steps are the same as the steps used to establish an MA PDU Session in Rel-17 specifications. The additions to support MP-DCCP-LL are further discussed below.

1a.	In the PDU Session Establishment Request the UE indicates (in the 5GSM capability) that it supports the MP-DCCP-LL steering functionality and, possibly, other steering functionalities such as MPTCP and ATSSS-LL.

1b.	The AMF selects an SMF supporting ATSSS.

5a.	In the SM Policy Control Create Request, the SMF includes the ATSSS Capabilities of the MA PDU Session (see TS 23.502 [3] and TS 29.512 [23]), which contain the MP-DCCP-LL capability and, possibly, other capabilities already defined, such as "MPTCP functionality with any steering mode and ATSSS-LL functionality with any steering mode", etc.

5b.	The PCF decides to allow the requested MA PDU Session and creates PCC rules containing MA PDU Session Control information (see TS 29.512 [23]), which specifies a steering functionality (e.g. MP-DCCP-LL, ATSSS-LL), a steering mode (e.g. Active/Standby), etc.

6a.	Based on the received PCC rules, the SMF creates N4 rules for the UPF including MAR rules for MP-DCCP-LL.

6b.	In the N4 Session Establishment Request, the SMF includes the N4 rules and indicates to UPF that MP-DCCP-LL Control Information should be provided (see next step). The MP-DCCP-LL Control Information indicates to UPF how many MP-DCCP connections are needed, which is determined from the received PCC rules map to QoS Flows. For example, if the SMF has two QoS Flows carrying traffic with steering functionality = MP-DCCP-LL, then the SMF indicates to UPF that two MP-DCCP connections are needed.

6c.	Based on the requested MP-DCCP-LL Control Information, the UPF provides the following MP-DCCP-LL parameters:

1)	Two "UE link-specific MP-DCCP-LL" IP addresses used only by the MP-DCCP-LL functionality in the UE, one associated with the 3GPP access and another associated with the non-3GPP access. It is possible that the UPF provides "UE link-specific MP-DCCP-LL" IP addresses that are not routable via N6 (e.g. IPv6 link-local addresses). For example, the "UE link-specific MP-DCCP-LL" IP addresses may be the following:

UE link-specific MP-DCCP-LL IP address over 3GPP access = 10.10.1.1

UE link-specific MP-DCCP-LL IP address over non-3GPP access = 10.10.2.1

NOTE 1:	The "UE link-specific MP-DCCP-LL" IP addresses are similar to the "UE link-specific multipath" IP addresses used for MPTCP, specified in TS 23.502 [3] and TS 29.244 [24].

NOTE 2:	In case MP-DCCP-LL and MPTCP are enabled for the same MA PDU Session, the "UE link-specific MP-DCCP-LL" IP addresses and the "UE link-specific multipath" IP addresses can be the same.

2)	"MP-DCCP Address Information" which contains (1) two "UPF link-specific MP-DCCP-LL" IP addresses for the UPF, one for each access and (2) one UPF port number per MP-DCCP connection.

7.	Based on the received PCC rules and the MP-DCCP-LL parameters received from UPF, the SMF creates the following information, which will be sent to the UE:

-	ATSSS rules;

-	QoS rules; and

-	"MP-DCCP Connection Setup Information" which contains information for the UE to setup the MP-DCCP connections with the UPF. For example, it indicates that MP-DCCP Connection #1 over 3GPP access should be established toward the UPF IP address 10.10.1.2 and UPF port 53671.

8a. In the PDU Session Establishment Accept, the ATSSS Container (defined in TS 24.193 [13]) contains:

1)	The ATSSS rules, which are applied by MDTC in the UE to route the traffic of the MA PDU Session across the DCCP connections; and

2)	The "network steering functionalities information" (see TS 24.193 [13]), which contains:

a.	The two "UE link-specific MP-DCCP-LL" IP addresses provided by UPF in step 6c; and

b.	The "MP-DCCP Connection Setup information" created by SMF in step 7.

11.	Based on the received "MP-DCCP Connection Setup information" the MDTC in the UE establishes one or more MP-DCCP connections with the MDTS in the UPF. The UE establishes the MP-DCCP connections immediately after receiving the PDU Session Establishment Accept message. The application protocol negotiated during each MP-DCCP connection will be decided by stage-3 (for example, it could be "mp-dccp-ll").

The MP-DCCP connections established between the MDTC in UE and the MDTS in UPF include also MP-DCCP connections associated with downlink-only QoS flows. This is achieved by sending "MP-DCCP connection setup information" to UE that includes information for establishing a MP-DCCP connection associated with a downlink-only QoS flow. The UE does not send uplink PDUs to a MP-DCCP connection associated with a downlink-only QoS flow, because the UE has no associated QoS rule for this MP-DCCP connection. However, the UE sends DCCP packets (e.g. Data-ACK, etc.) over this MP-DCCP connection based on the normal MP-DCCP protocol operation.

### 6.3.4	MA PDU Session Modification procedure

The MA PDU Session Modification procedure (either network-requested or UE-requested) may be used to add or remove QoS flows from an established MA PDU Session, as specified since Rel-16.

After the MA PDU Session Modification procedure is completed, then:

-	If a QoS flow is deleted and this QoS flow was used to transfer MP-DCCP-LL traffic, then the MP-DCCP connection associated with this QoS flow (one DCCP connection per access) is released. The UE receives updated "MP-DCCP Connection Setup Information" in order to determine which MP-DCCP connection to release.

-	If a QoS flow is created and this QoS flow will be used to transfer MP-DCCP-LL traffic, then the MP-DCCP connection associated with this QoS flow (one DCCP connection per access) is established between the UE and UPF. The UE receives updated "MP-DCCP Connection Setup Information" in order to determine the UPF IP addresses and ports for the new MP-DCCP connections.

### 6.3.5	Example of MP-DCCP-LL Operation

To better explain the MP-DCCP-LL operation, we consider an example in this clause. In this example, it is assumed that the PCF creates two PCC rules which contain steering functionality = MP-DCCP-LL (see Figure 6.3.5-1 below) and the UPF provides the following MP-DCCP-LL parameters:

1)	"UE Link-specific MP-DCCP-LL" IP addresses:

UE Link-specific MP-DCCP-LL IP address over 3GPP access = 10.10.1.1

UE Link-specific MP-DCCP-LL IP address over non-3GPP access = 10.10.2.1

2)	MP-DCCP-LL Address Information:

UPF Link-specific MP-DCCP-LL IP address for 3GPP access = 10.10.1.2

UPF Link-specific MP-DCCP-LL IP address for non-3GPP access = 10.10.2.2

UPF Port for MP-DCCP Connection #1 = 53671

UPF Port for MP-DCCP Connection #2 = 53672

Based on the above MP-DCCP-LL Address Information, the SMF creates the following " MP-DCCP Connection Setup information", which is sent to UE.

-	MP-DCCP Connection #1:

-	UPF Link-specific MP-DCCP-LL IP address for 3GPP access: 10.10.1.2

-	UPF Link-specific MP-DCCP-LL IP address for non-3GPP access: 10.10.2.2

-	UPF Port: 53671

-	Associated QFI: QFI-x

-	Associated Steering Mode: Active/Standby

-	MP-DCCP Connection #2:

-	UPF Link-specific MP-DCCP-LL IP address for 3GPP access: 10.10.1.2

-	UPF Link-specific MP-DCCP-LL IP address for non-3GPP access: 10.10.2.2

-	UPF port: 53672

-	Associated QFI: QFI-y

-	Associated Steering Mode: Smallest Delay

The UE applies the received "UE link-specific MP-DCCP-LL" IP addresses and the " MP-DCCP Connection Setup information" and establishes two MP-DCCP connections with the UPF over both 3GPP access and non-3GPP access, as shown below.

Figure 6.3.5-1: Example of MP-DCCP-LL Operation

After the two MP-DCCP connections are established between the UE and the UPF, the MP-DCCP-LL routes the PDUs received from the upper layers to one of the flows of a MP-DCCP connection, as illustrated in Figure 6.3.5-2 and Figure 6.3.5-3 below for up-link and down-link traffic, respectively.

First, the QoS rules are applied on a received PDU (e.g. IP packet or Ethernet frame) to determine a QFI for this PDU. Subsequently, the MDTC finds a matching ATSSS rule for the PDU based on which it identifies the steering mode that should be applied for this PDU. Finally, the MDTC selects a MP-DCCP connection for the PDU based on the selected QFI and the steering mode. For each MP-DCCP Connection, the MDTC maintains an associated steering mode and QFI according on the received " MP-DCCP Connection Setup information". Finally, the PDU is encapsulated in a DCCP datagram and is added to a DCCP/IP packet destined to [UPF Link-specific MP-DCCP-LL IP address = 10.10.1.2, UPF Port = 53671].

NOTE 1:	The QFI selected for a PDU is transferred down to the selected access (3GPP or non-3GPP) so that the selected access can transfer the PDU via the corresponding QoS flow. For a packet created by the MP-DCCP protocol itself (e.g. a Data ACK), the QFI delivered to the access is the QFI associated with the MP-DCCP connection over which this packet is transmitted.

NOTE 2:	The header compression performed by the MP-DCCP Tunnel Client and the MP-DCCP Tunnel Server (see clause 6.3.2) is not shown in the following two figures for simplicity.

Figure 6.3.5-2: Example of user-plane operation with MP-DCCP-LL (UL direction)

Figure 6.3.5-3: Example of user-plane operation with MPDCCP-LL (DL direction)

### 6.3.6	Support of Steering Modes

MP-DCCP solution supports all specified steering modes in TS 23.501 [2]. In addition, MP-DCCP can support additional steering modes as listed in clause 6.3.1.

MP-DCCP protocol may provide additional modes by new scheduling algorithms and by flexibly combining any of the existing steering modes.

### 6.3.7	Support of Re-Ordering

Traffic splitting over access paths with different characteristics causes scrambling of the original data stream. A reassembly of such data stream requires a re-ordering. .

MP-DCCP does path sequencing, connection sequencing and has inherent latency information, i.e. all properties that are necessary for the packet receiving side, i.e. UE or UPF, to be able to do proper re-ordering. Different re-ordering strategies may be used which can be achieved by using implementation specific means.

### 6.3.8	Impacts on services, entities, interfaces and IETF protocols

The ATSSS rules and the N4 rules are enhanced to support values for the MP-DCCP-LL steering functionality and for potentially new steering modes. The UE and the UPF receive information (see the " MP-DCCP Connection Setup information") which indicates how many MP-DCCP connections should be established between the UE and the UPF, and the QoS flow and steering mode associated with each MP-DCCP connection.

IETF protocols

-	The MP-DCCP-LL solution is based on the following draft specifications defined by IETF. The MP-DCCP-LL does not require any changes to these specifications.

RFC 4340 [11], draft-ietf-tsvwg-multipath-dccp [12].

AMF

-	No impact. It is assumed that if 5GC supports ATSSS / Rel-18, then all ATSSS-capable SMFs in 5GC are capable of supporting MP-DCCP-LL.

SMF

-	From the PCC rules, it shall determine the number of MP-DCCP connections needed.

-	Shall indicate to UPF the number of MP-DCCP connections needed.

-	Shall create and send to UE the " MP-DCCP Connection Setup Information" based on the MP-DCCP-LL Address Information received from UPF.

-	From the received PCC rules, it shall create corresponding ATSSS rules and QoS rules for the UE.

-	From the received PCC rules, it shall create corresponding N4 rules (PDRs, MAR, QER, etc.) for the UPF.

PCF

-	Shall be able to create PCC rules using the MP-DCCP-LL steering functionality.

UPF:

-	Shall be able to allocate the "UE Link-specific MP-DCCP-LL" IP addresses.

-	Shall be able to allocate MP-DCCP-LL Address Information, i.e. two IP addresses used for MP-DCCP-LL (one per access) and one UDP port number for each MP-DCCP connection.

-	Shall apply the N4 rules (e.g. PDRs and associated MARs) to select a MP-DCCP connection and an access (sub-flow) on this connection, for each DL PDU. Each MAR using the MP-DCCP-LL steering functionality shall identify a MP-DCCP Connection.

-	Shall select a MP-DCCP connection for an DL PDU based on the steering mode and QoS flow selected with this PDU.

-	Shall be able to support the DCCP protocol with multipath extensions.

-	Shall apply the QoS Enforcement Rules (QERs) to map the traffic of each MP-DCCP connection to a QoS flow.

UE:

-	Shall be able to indicate support of MP-DCCP-LL when requesting a MA PDU Session.

-	Shall establish N MP-DCCP connections to UPF via each access, based on the received " MP-DCCP Connection Setup Information".

-	Shall apply the ATSSS rules to select a steering mode for each UL PDU.

-	Shall select a MP-DCCP connection for an UL PDU based on the steering mode and QoS flow selected with this PDU.

-	Shall be able to support the DCCP protocol with multipath extensions.

-	Shall apply the QoS rules to map the traffic of a SDF to a QoS flow.

## 6.4	Solution #3.2: Redundant traffic steering triggered by AF

### 6.4.1	Introduction

This solution aims at addressing key Issue #3 about support of redundant traffic steering. In particular, this solution mainly focuses on whether and how to introduce mechanisms to active and deactivate packet duplication in a dynamic manner, e.g. triggered by network entities or application functions. For how the UE and UPF apply the redundancy steering mode, Solution #3.1 specified in clause 6.1 can be applied.

### 6.4.2	High-level Description

This solution aims at addressing key Issue #3 about support of redundant traffic steering. In particular, this solution mainly focuses on whether and how to introduce mechanisms to activate and deactivate packet duplication in a dynamic manner, e.g. triggered by network entities or application functions. For how the UE and UPF apply the redundancy steering mode, Solution #3.1 specified in clause 6.1 can be applied.

Application function (AF) can provide packet loss rate threshold value for specific traffic flows to the PCF via the NEF, triggering PCF to update the PCC rules related to the traffic flows, e.g. using redundant steering mode to transmit the traffic flows with applying the packet loss rate threshold value. When the SMF receives the updated PCC rules provided by the PCF, the SMF updates the N4 rules to UPF and ATSSS rules to UE via AMF respectively. The UE and the UPF transmit the traffic flows with redundancy steering mode as defined in Solution #3.1 based on the updated ATSSS rules and N4 rules.

### 6.4.3	Procedures

#### 6.4.3.0	General

Application function (AF) can provide packet loss rate threshold value for specific traffic flows to PCF via the NEF as defined in the clauses 4.3.6.2, 4.15.6.6 or 4.15.6.6a of TS 23.502 [3]. When the parameter (i.e. the packet loss rate threshold value) is updated to PCF, the PCF may decide to trigger redundant traffic steering by updating PCC rules including the updated parameters to the SMF, enabling the SMF modifies the N4 rules and ATSSS rules to UPF and UE respectively. Clause 6.4.3.1 illustrates that the PLR threshold value is provided to PCF via the procedure of AF requesting to influence traffic routing for Sessions not identified by an UE address, while the clause 6.4.3.2 illustrates that the PLR threshold value is provided to PCF via the AF session with required QoS update procedure.

Editor's note:	It is FFS which one of the options is be selected.

#### 6.4.3.1	Procedure of AF providing PLR threshold value via Nnef_TrafficInfluence_Create/Update operation service

Figure 6.4.3.1-1: Procedure of AF providing PLR threshold value via Nnef_TrafficInfluence_Create/Update operation service

1.	Step 1 to step 5 in Figure 4.3.6.2-1 of TS 23.502 [3] are performed with following modification:

-	The request message in step 1, step 2 and step 4 includes the maximum loss rate tolerated by the application corresponding to the traffic flow identified by the traffic filtering info.

-	In step 5, the PCF determines to updates the SMF with corresponding new policy information about the MA PDU Session based on the maximum loss rate received in step 4. The MA PDU Session control information included in the PCC rules indicate that redundant steering mode with PLR threshold value is applied to traffic flows identified by the traffic filtering information.

2.	SMF derives the updated N4 rules and ATSSS rules based on the updated PCC rules. SMF sends the N4 rules to UPF via N4 Session modification procedure. Both N4 rules and ATSSS rules indicate that when transmitting the specific traffic flows identified by the traffic descriptor, the redundant steering mode shall be applied. The redundant steering mode can be triggered dynamically as specific in clause 6.1 in Solution #3.1.

3.	SMF sends Namf_Communication_N1N2MessageTransfer message to AMF, which include N1 SM container that consists of the ATSSS rules.

4.	AMF sends the ATSSS rules to the UE via NAS message.

When both UE and UPF obtain the ATSSS rules and N4 rules respectively, they transmit the specific traffic flows by using redundant steering mode as defined in Solution #3.1.

#### 6.4.3.2	Procedure of AF providing PLR threshold value via Nnef_AFsessionWithQoS_Create/Update operation service

Figure 6.4.3.2-1: Procedure of AF providing PLR threshold value via Nnef_AFsessionWithQoS_Create/Update operation service

1.	Step 1 to step 5 in figure 4.15.6.6-1 or 4.15.6.6a-1 of TS 23.502 [3] are performed with following modifications:

-	The request message in step 1 and 3 includes the QoS parameter of PLR corresponding to the traffic flow identified by the Flow descriptions.

-	the TSCTSF is not involved in this procedure, so the steps 3a, 3b, 4b, 6a, 7a and 7b are not applicable.

2.	the PCF determines to update the SMF with corresponding new policy information about the MA PDU Session based on the PLR threshold value received in step 1. The MA PDU Session control information included in the PCC rules indicate that redundant steering mode with PLR threshold value is applied to traffic flows identified by the traffic filtering information.

3.	SMF derives the updated N4 rules and ATSSS rules based on the updated PCC rules. SMF sends the N4 rules to UPF via N4 Session modification procedure. Both N4 rules and ATSSS rules indicate that when transmitting the specific traffic flows identified by the traffic descriptor, the redundant steering mode shall be applied. The redundant steering mode can be triggered dynamically as specific in clause 6.1 in Solution #3.1.

4.	SMF sends Namf_Communication_N1N2MessageTransfer message to AMF, which include N1 SM container that consists of the ATSSS rules.

5.	AMF sends the ATSSS rules to the UE via NAS message.

When both UE and UPF obtain the ATSSS rules and N4 rules respectively, they transmit the specific traffic flows by using redundant steering mode as defined in Solution #3.1.

### 6.4.4	Impacts on Existing Nodes and Functionality

PCF impact:

-	Ability to determine to apply redundant steering mode with PLR threshold value based on the parameters provided by AF.

## 6.5	Solution #3.3: New traffic duplication steering mode

### 6.5.1	Introduction

This new solution addresses KI#3 on Support of redundant traffic steering.

### 6.5.2	High-level Description

With this solution redundant traffic steering is enabled as a new steering mode. The new steering mode enforces, based on certain criteria, that a Packet Data Unit (PDU) is duplicated and sent to the other access. Traffic duplication can be activated and de-activated by setting or re-setting the traffic duplication criteria.

Editor's note:	It is FFS whether the UE needs to inform the network about its capability to support traffic duplication steering mode.

Traffic duplication reduces end-to-end latency and packet loss rate, but at the cost of utilizing resources over both accesses. Thus, selective duplication is a necessity to avoid wasting unnecessarily network, and especially radio resources. The state of each access is dynamically changing over time, and hence a static selection of an access as primary access is suboptimal.

The new steering mode operates on a per PDU basis in the following way: First, for each PDU the access to be used as primary access is determined either statically, or dynamically based on access conditions, e.g. smallest delay.  Second, once the primary access for the PDU is selected and if the duplication criteria are met, the PDU is duplicated and sent over the other (the current secondary) access.

As part of the ATSSS Rule and MA Rule at SDF level, the UE and the UPF respectively receive the necessary information to use the new steering mode.

In certain cases, duplication of traffic may be desired only for a specific access, e.g. limited to the non-3GPP access to avoid excessive charging. For this purpose, an Access Constraint Indicator can be provided indicating the access to which duplication is not allowed, i.e. it can be set to 3GPP or non-3GPP. If not set, duplication can be applied for any access.

1.	Regarding the selection of the primary access, the following configuration parameter can be set:

a.	Static: One access is indicated and set as the primary access. The other access is the secondary access used for duplication.

b.	Dynamic: Based on measured performance criteria (e.g. PLR, RTT), the best-performing access is selected as primary access, and the other access is used as secondary access.

2.	If none of the duplication criteria described below is provided and the Access Constraint Indicator is not set, all the traffic sent to the primary access is duplicated to the secondary access. If duplication criteria are provided, (part of the) traffic is duplicated to the other access whenever the duplication criteria are met, and the secondary access is available.

The new steering mode is applicable to GBR and non-GBR traffic as it generally improves the user experience. For GBR traffic networks resources need to be reserved on both accesses.

Editor's note:	How the receiver will treat duplicated packets in case of MP-QUIC or MP-DCCP is FFS.

If present, traffic duplication criteria are evaluated by the UE for the UL and by the UPF for the DL to decide which packets need to be duplicated. One or more of the following traffic duplication criteria can be applied to the traffic on the primary access:

1.	Link quality: Traffic duplication is started or stopped based on link quality measurements, e.g. considering Packet Loss Rate (PLR) or Round Trip Time (RTT) thresholds. The measurement result focuses only on the performance of a single link, i.e. the 3GPP or non-3GPP link in UL or DL. When the measured link performance violates the provisioned threshold, the next packets will be duplicated to the other path, otherwise the traffic will be sent on the currently selected primary access. This allows to limit duplication of traffic to the case where the link quality decreases, thus the probability to lose packets increases. These duplication threshold values can be specified in a similar way to existing Threshold values.

The link quality criterion allows to specify the duplication conditions for the selected primary access.

Duplication threshold values: One or more threshold values may be provided for a steering mode. A threshold value may be either a value for RTT or a value for PLR. The threshold values are applicable to currently selected primary access and are applied by the UE and UPF to decide whether a PDU shall be duplicated and send on the other access.

Applicability: This criterion keeps track of the status of the primary access path and enables the UE and the UPF to independently apply duplication when considered useful based on the measurements available. Thus, it can tackle temporary degradation of an access path in UL and/or DL.

2.	Percentage of duplicated traffic (Max or fixed percentage is possible): The max percentage variant, if indicated, allows to define a duplication budget as the max percentage (e.g. up to 20%) of traffic that can be duplicated over a certain time. It is up to implementation which traffic is duplicated. The fixed percentage variant, if indicated, mandates the exact percentage of traffic that should be duplicated to the secondary access. For example, fixed 20% duplication rate would mean that UE and UPF duplicate 20% of the packets, e.g. every 5th packet. A max of 20% duplication rate would mean that UE and UPF duplicate every 5th or less packet over a certain time period.

Applicability: Unless fixed percentage is indicated, this criterion enables the UE and the UPF to select how much and which of the traffic to duplicate based for example on locally available information such as the battery state of the UE or other possible criteria, such as:

i.	Important PDUs: Traffic duplication is applied based on header information indicating whether a PDU is important or not, i.e. only important PDUs are duplicated, and up to the maximum duplication budget. For example, duplicating only PDUs carrying I-frames, and not P-frames (which is indicated in a RTP header extension), in video streams, or duplicating only video and not audio in a video surveillance application.

NOTE 1:	The concrete information in the PDU header that is used for traffic duplication is not specified.

ii.	Survival time: Survival Time, as defined in TS 23.501 [2], refers to the time period an application can survive without any data burst (with a certain burst size). As per TSC feature, Survival Time is part of TSCAI (see TS 23.501 [2]). The AF may provide the time an application can survive without any burst. It refers to the time that an application consuming a communication service may continue without an anticipated message and can be expressed as the maximum number of messages or in terms of time units.

NOTE 2:	The exact usage of the Survival Time in the TSCAI information is not specified, but the Survival Time has to be included as part of the ATSSS and MA rules to be used by the UE and the UPF respectively.

Essentially the two criteria are easily combinable since:

1.	the first (Link quality) controls the dynamic activation of duplication of traffic

2.	the second (Percentage), upon activation determines which PDUs should be duplicated based on the indicated budget.

Access selection is based on relative performance of the two accesses, and by selecting always the best one it reduces the need for duplication. On the other hand, Link quality criterion for duplication captures the temporal evolution of access state and relies on absolute performance values. The combination of dynamic primary access selection and partial duplication significantly reduces the duplication overhead since the duplication is applied only when needed and to the extent it is needed.

The duplication steering mode, together with duplication criteria, needs to be authorized by the PCF in the PCC Rules, and the SMF provides an indication for duplication and the duplication criteria in the ATSSS Rule to the UE, and in the Multi-Access Rule (MAR) to the UPF. After receiving the rules, UE and UPF determine when and how to duplicate traffic based on the provided duplication criteria and the measured performance of each access.

The AF can also request to start and stop traffic duplication by including the corresponding duplication steering mode indication and traffic duplication criteria to be applied for the application traffic. The NEF provides an API allowing the AF to request start and stop of traffic duplication and providing traffic duplication criteria.

The duplication steering mode requires that the Multi-Access Rule (MAR) and the ATSSS Rule, both generated by the SMF will need to be updated as follows:

Table 6.5.2-1: Updated structure of ATSSS Rule and MA Rule

| Information name | Description | Category | SMF permitted to modify in a PDU context | Scope |
| --- | --- | --- | --- | --- |
| Static primary access (NOTE 1) | Identifies the static primary access (e.g. 3GPP access) | Optional | Yes | PDU context |
| Static secondary access (NOTE 1) | Identifies the static secondary access (e.g. non-3GPP access) | Optional | Yes | PDU context |
| Dynamic primary access (NOTE 2) | Identifies the performance criteria (PLR, RTT) to determine the primary access | Optional | Yes | PDU context |
| Access Constraint Indicator | Identifies the accesses to which duplication is not allowed. If not set, duplication is allowed on all accesses. | Optional | Yes | PDU context |
| Duplication criterion | Identifies the duplication criterion that should be applied and associated parameters (see below) | Optional | Yes | PDU context |
| Duplication threshold values | A Maximum RTT and/or a Maximum Packet Loss Rate. | Optional | Yes | PDU context |
| Survival time | A Maximum duration that, as defined in TS 22.261, is synonymous with the time period an application can survive without any data burst. | Optional | Yes | PDU context |
| Percentage of duplicated traffic | A Maximum or fixed percentage of traffic that can be duplicated plus an indication whether max or fixed percentage is assumed. | Optional | Yes | PDU context |
| NOTE 1: Static primary and Static secondary access are configured together. NOTE 2: Dynamic primary access excludes configuration of Static primary and Static secondary access. |  |  |  |  |

### 6.5.3	Procedures

Editor's note:	This clause describes the procedures and information flows for the solution.

### 6.5.4	Impacts on Existing Nodes and Functionality

The ATSSS rules and the N4 rules are enhanced with a new steering mode definition and with new parameters.

SMF

-	Based on the PCC rules, create ATSSS rules and N4 rules with the new steering mode data including optional traffic duplication criteria.

PCF

-	Provide PCC rules considering new steering mode including optional traffic duplication criteria to the SMF.

UPF:

-	Based on the traffic duplication steering mode and optionally available traffic duplication criteria decide when to duplicate traffic via the available access paths of a MA PDU session for the downlink traffic.

UE:

-	Based on the traffic duplication steering mode and optionally available traffic duplication criteria decide when to duplicate traffic via the available access paths of a MA PDU session for the uplink traffic.

AF:

-	AF can request start and stop of traffic duplication for an application session and optionally provide traffic duplication criteria which allow UPF and UE to decide when duplication of traffic is required.

NEF:

-	NEF provides an API allowing the AF to request start and stop of traffic duplication and providing traffic duplication criteria.

## 6.6	Solution #3.4: Redundant steering mode with duplication information and trigger mechanisms

### 6.6.1	Introduction

KI#3 indicates the need to study a new steering mode in order to enable the transmission of redundant traffic between UE and UPF. The solution proposed in this clause enables the CN to indicate to the UE and to the UPF what percentage of the traffic needs to be duplicated and over which access link. In addition, the solution defines mechanisms that allow the CN to trigger the establishment of a MA PDU Session with Redundant Steering Mode (RSM) to enable its usage in case some QoS requirements (e.g. PER/reliability) cannot be met with a regular (single access) PDU Session.

NOTE:	The working of RSM relies on the support of redundant packet transmission from the underlying protocol e.g. MPTCP, MPQUIC, or MPCCP.

### 6.6.2	High-level Description

#### 6.6.2.1	Provision of RSM related parameters to UE and UPF

The provision of the redundant steering mode (RSM) parameters is based on the following principles:

-	The PCF, based on the QoS requirements from the AF, will generate PCC rules to be sent to the SMF indicating that the redundant steering mode (RSM) is to be used for a certain traffic and/or application.

-	Based on the PCC rules, the SMF decides that the redundant steering mode (RSM) needs to be used for a certain traffic and/or application. The SMF also decides how much such traffic needs to be duplicated and over which access path it is to be duplicated.

-	The SMF indicates to the UE and to the UPF via ATSSS rules and N4 rules, respectively, the details of the RSM:

-	indication of which traffic and/or application is subject to RSM.

-	how much of that traffic needs to be duplicated (duplication factor): this may vary between 0 to 100%. If 0% is indicated, it means that no duplication takes place and that all the traffic is sent only over the primary access link. If, e.g. 20, 50 or 100% is indicated, it means that that only 20, 50 or 90% of the traffic sent over the primary access link is duplicated over the secondary access link. 100% means that all traffic is duplicated over the secondary access link. The indication can be different for UL and DL. For example, there could be a duplication factor of 50% in UL and of only 30% in DL.

-	indication of which of the access paths is the secondary access. If secondary access = 'non-3GPP access', then 100% of the traffic is sent over the 3GPP access path and the X% duplicated PDUs (indicated by the duplication factor) will be sent over the non-3GPP access path. The indication can be different for UL and DL.

NOTE:	If the duplication factor is not indicated, it is implicitly assumed that all the traffic is duplicated.

Editor's Note: How UPF/UE treat duplicated packets is FFS.

#### 6.6.2.2	Triggering the usage of a MA PDU Session with RSM

In addition, in scenarios in which the QoS requested for one or more QoS flows of a single access PDU Session cannot be met (e.g. a too high PER), the SMF may decide, based on information provided by the RAN and/or implementation dependent mechanisms, to use, instead use the regular single access PDU Session, a MA PDU Session with RSM to match the QoS requirements from the AF.

In order to do this, two options are possible:

-	Option 1: UE proactively indicating the possible use of a MA PDU Session

If the UE can proactively indicate its willingness to upgrade to a MA PDU Session, then the UE Requested PDU Session Establishment with Network Modification to MA PDU Session procedure can be reused. The UE sends its ATSSS rules with the MA PDU Network-Upgrade Allowed indication and the SMF can trigger the switch to the MA PDU Session with RSM according to the indicated ATSSS rules.

-	Option 2: SMF asking UE to upgrade to a MA PDU Session

In that case, UE initially triggers the establishment of a regular single access PDU Session Establishment procedure without indicating ATSSS rules nor MA PDU Session Request. In the PDU Session Establishment Accept the SMF may indicate to the UE that if it establishes a MA PDU Session the provided QoS level will increase. The UE, after receiving such indication, may decide to upgrade to a MA PDU session by modifying the single access PDU Session and sending its ATSSS capabilities together with the MA PDU Request indication.

### 6.6.3	Procedures

Provision of rules to SMF, UE and UPF

The solution is based on the reuse of the existing procedures for ATSSS to provision the necessary rules to SMF, UE and UPF:

-	the PCF provides the extended PCC rules (see clause 6.x.2) to the SMF at, e.g. UE triggered Service Request (see TS 23.502 [3] clause 4.2.3.2);

-	the SMF provides the extended ATSSS and N4 rules to the UE and the UPF, respectively, by means of, the MA PDU Session Establishment procedure (see TS 23.502 [3] clause 4.22.2).

Triggering of usage of RSM by the SMF

In order to trigger the establishment of the MA PDU Session:

-	Option 1:

This option re-uses the UE Requested PDU Session Establishment with Network Modification to MA PDU Session (see clause 4.22.3 of TS 23.502 [3]) with a "MA PDU Network-Upgrade Allowed" indication. The SMF may decide to establish the MA PDU Session based on the fact that the QoS characteristics levels of a certain QoS flow of the PDU Session that is being establish do not meet the QoS requirements from the AF. The SMF will then sends the appropriate ATSSS rules and N4 rules to the UE and to the UPF, respectively, to initiate the usage of the RSM.

-	Option 2:

This option is based on the extension of the following procedures.

Extend the procedure for setting up an AF session with required QoS (see clause 4.15.6.6 of TS 23.502 [3]) with the following changes:

-	Step 1: instead of using UE's IP address, the AF indicates that the UE's GPSI to the NEF when it provides the related QoS requirements.

NOTE 2:	It is assumed that the AF knows (e.g. based on application level subscription) when to provide the QoS requirements for the UE even the PDU Session is not yet established.

Editor's note:	How to identify specific PDU session, e.g. based on DNN/S-NSSAI, is FFS.

-	Step 3:

The NEF temporarily stores the QoS requirements associated to the UE's GPSI and a flag associated to the GPSI is added in the UDM. Based on that flag, at PDU session establishment, the SMF contacts the NEF to retrieve the QoS requirements.

Editor's note:	Alternative procedures and details of the flag in the UDM are FFS.

Extend the UE Requested PDU Session Establishment (see clause 4.3.2.2.1 of TS 23.502 [3]) with the following changes:

-	Step 11: the SMF is aware that it the QoS requirements for a certain QoS flow of the PDU Session that is being established cannot be met. Because of that the QoS flow is established with lower QoS characteristics levels.

-	After step 17: the SMF sends a PDU Session Modification Command message to the UE including an indication that, if the UE switches to MA PDU Session, then the QoS levels will be improved.

After the UE receives the indication from the SMF and the single access PDU Session is established, the UE may decide to modify the PDU Session into a MA PDU Session by sending its ATSSS capabilities and a MA PDU Request indication. The SMF will then provide the appropriate ATSSS rules and N4 rules to the UE and the UPF, respectively, to initiate the usage of the RSM and improve the QoS provided to the UE.

### 6.6.4	Impacts on Existing Nodes and Functionality

PCF:

-	Extension of PCC rules to include RSM as a possible steering mode.

NEF:

-	Optional: Receives from AF per-GPSI QoS Requirements and temporarily stores them.

-	Optional: Stores in UDM flag to indicate to SMF to contact NEF at PDU session establishment and retrieve QoS requirements.

SMF:

-	Extension of PCC rules/ATSSS rules/N4 rules to include RSM as a possible steering mode.

-	Extension of ATSSS rules/N4 rules to include indication of how much traffic needs to be duplicated and over which access path.

-	Decides to trigger the establishment of a MA PDU Session with RSM based on QoS conditions.

-	Optional: check UDM for flag to retrieve QoS Requirements from NEF at PDU Session establishment.

UE:

-	Extension of ATSSS rules to include indication of RSM, of how much traffic needs to be duplicated and over which access path.

-	Receives indication from SMF to switch to MA PDU Session to improve QoS level of QoS Flow.

UPF:

-	Extension of N4 rules to include indication of RSM, of how much traffic needs to be duplicated and over which access path.

UDM:

-	Optional: Stores flag to retrieve QoS Requirements from NEF at PDU Session establishment.

AF:

-	Optional: Needs to provide per-GPSI QoS Requirements to 5GC before PDU Session Establishment (Nnef_AFsessionWithQoS_Create service operation).

## 6.7	Solution #5.1: Support traffic switching between two non-3GPP paths

### 6.7.1	Introduction

This solution addresses KI#5 on support traffic switching between one non-3GPP access path from the UE to a N3IWF in a PLMN and another non-3GPP access path from the UE to a TNGF in the same PLMN.

### 6.7.2	High-level Description

In this solution, it is assumed that the two registrations via two non-3GPP access in the same PLMN are performed in order to enable switching the traffic from a source non-3GPP access path to a target non-3GPP access path and after switching the traffic, only one UE registration via non-3GPP access may exist.

When UE decides to switch traffic of MA PDU Session from source non-3GPP access (e.g. trusted non-3GPP access) path to a target non-3GPP access (e.g. untrusted non-3GPP access) path in the same PLMN, UE shall perform registration procedure via the target non-3GPP access (untrusted non-3GPP access) path.

The MOBIKE protocol may not be able to support handover procedure between trusted non-3GPP access and untrusted non-3GPP access and vice versa due to the simultaneous changes of the IP address of the two end point of IKEv2, i.e. the UE change of local IP address from source N3GPP access to target N3GPP access and the simultaneous change of GW endpoint from N3IWF to a TNGF.

N3IWF/TNGF shall select the same AMF based on AN parameters provided by UE.

AMF shall update UDM with the RAT type of target non-3GPP access after the registration procedure over the target non-3GPP access path is completed.

When adding user-plane resources over the target non-3GPP access path, SMF may update ATSSS rules and N4 rules to the UE and the UPF respectively to indicate that the traffic have to be send on the target N3GPP access.

### 6.7.3	Procedures

This clause specifies how a UE can handover one leg of MA PDU Session from a source non-3GPP access to a target non-3GPP access path. Figure 6.7.3-1 illustrates the procedure of switching traffic of a MA PDU Session from trusted non-3GPP access path to untrusted non-3GPP access path. It can also be applied to the procedure of switching traffic of a MA PDU Session from untrusted non-3GPP access path to trusted non-3GPP access path by replacing the TNAP/TNGF with untrusted non-3GPP access network/N3IWF, and vice versa.

Figure 6.7.3-1: Switching traffic of a MA PDU Session from trusted non-3GPP access path to untrusted non-3GPP access path

0.	UE registers to 5GC via trusted non-3GPP access and has established a MA PDU session over trusted non-3GPP access.

1.	The UE shall initiate Registration procedure via untrusted non-3GPP access using step 1 to step 4 in clause 4.12.2.2-1 of TS 23.502 [3].

2.	UE sends the IKE_AUTH request which including an EAP-Response/5G-NAS packet that contains the Access Network parameters (AN parameters) and the Registration Request message. The AN parameters shall contain the same information used in the registration procedure over trusted non-3GPP access including the GUAMI, the Selected PLMN ID, the Requested NSSAI (if included) and the Establishment cause. The Registration Request message shall include the non-3GPP Path Switching indication, which indicate that the UE intends to switch traffic between non-3GPP accesses.

3.	The N3IWF shall select the same AMF that TNGF selected via trusted non-3GPP access based on the received AN parameters. The N3IWF shall then forward the Registration Request received from the UE to the selected AMF within an N2 message, which also contains N2 parameters that include the Selected PLMN ID and the Establishment cause.

4.	The AMF shall send a NAS Security Mode Command to UE in order to activate NAS security. The UE then replies the NAS Security Mode Complete message within an EAP/5G-NAS packet as specified in step 9 in clause 4.12.2.2-1 of TS 23.502 [3].

NOTE 1:	It is assumed that there is no need to perform authentication over untrusted non-3GPP access since security context is available in the selected AMF.

5.	The AMF shall send an NGAP Initial Context Setup Request message that includes the N3IWF key.

6.	UE perform registration procedure via untrusted non-3GPP access as specified in step 11 to step 13 in clause 4.12.2.2-1 of TS 23.502 [3].

7.	When the registration is completed over trusted non-3GPP access, AMF shall update the RAT type of untrusted non-3GPP access with the UDM using Uudm_UECM_Update service. AMF shall also initiate a deregistration timer over the untrusted non-3GPP access. The usage of this deregistration timer is to deregister UE over untrusted non-3GPP access without releasing UE context of source (i.e. trusted) non-3GPP access when the deregistration timer is expired and the UE has not completed non-3GPP path switching.

8.	UE adds user-plane resources over untrusted non-3GPP access with the same PDU Session ID of the established MA PDU Session as specified in clause 4.22.7 of TS 23.502 [3] with following clarifications and additions:

-	When adding user-plane resources over untrusted non-3GPP access, SMF shall include additional indication (e.g. RAT type) in Namf_Communication_N1N2MessageTransfer to indicate AMF that N2 SM information included in this message should be sent over untrusted non-3GPP access.

-	The N4 rules are updated to UPF to indicate that the traffic have to be send on the target N3GPP access.

-	The MA PDU Session Establishment Accept message received by the UE may contain updated ATSSS rules which can be applied when there are two non-3GPP access paths available. The updated ATSSS rules shall indicate that the traffic shall be send on the target non-3GPP access path.

9.	If the User Plane of the MA PDU Session is activated in untrusted non-3GPP access, the SMF releases of resources over trusted non-3GPP access as specified in clause 4.22.10 of TS 23.502 [3] with following clarifications and additions:

-	When releasing user-plane resources over trusted non-3GPP access, SMF shall include additional indication (e.g. RAT Type) in Namf_Communication_N1N2MessageTransfer to indicate AMF that N2 SM information included in this message should be sent over trusted non-3GPP access.

-	After the resources over trusted non-3GPP access are completely released, the user plane resources of single access PDU session over this access will also be deactivated.

10.	If the deregistration timer over untrusted non-3GPP access is expired and AMF is not informed of the completion of non-3GPP access path switching of the UE (i.e. user plane resources over trusted non-3GPP access is released), AMF shall perform deregistration procedure via untrusted non-3GPP access as specified in clause 4.2.2.3.3 of TS 23.502 [3]

### 6.7.4	Impacts on Existing Nodes and Functionality

UE:

-	Performs Registration with new registration type.

-	Temporarily maintains simultaneous parallel user plane tunnel over trusted non-3GPP access and untrusted non-3GPP access.

-	When the UE receives the N2 resources Registration Accept message for the target non-3GPP access, the UE shall add user plane resources over target non-3GPP access.

AMF:

-	Initials deregistration timer over target non-3GPP access when registration with the new registration type is completed.

-	Update the RAT type of target non-3GPP access with the UDM by using Nudm_UECM_Update service after registration is completed.

-	Perform Deregistration when deregistration timer is expired and the AMF is not informed of the completion of non-3GPP access path switching of the UE.

SMF:

-	Indicates to AMF which access path should the N2 SM information be sent.

-	Update N4 rules and ATSSS rules when UE requests to switch traffic between non-3GPP accesses.

UPF:

-	Temporarily maintains simultaneous parallel user plane tunnel over trusted non-3GPP access and untrusted non-3GPP access.

UDM:

-	Update the RAT type of target non-3GPP access by using Nudm_UECM_Update service.

## 6.8	Solution #5.2: Delaying UDM Registration until non-3GPP access switching completes

### 6.8.1	Introduction

This solution addresses Key Issue #5 "Switching traffic of an MA PDU Session between two non-3GPP access paths" by allowing two simultaneous registrations over two non-3GPP accesses. In this solution, in order to minimize overall system impact, the AMF delays UDM Registration until access switching is completed.

### 6.8.2	High-level Description

When a UE establishes MA PDU Session, the SMF may indicate to the UE whether non-3GPP access switching is supported. Based on this indication, the UE may determine to change non-3GPP access leg. Whether and when to switch non-3GPP access is determined by the UE. If the UE determines to switch access, the UE performs registration over the new non-3GPP access with new registration type to indicate the registration is for switching non-3GPP access.

The AMF follows normal registration procedure as described in TS 23.502 [3] but does not perform UDM registration. After the Registration procedure is completed, the UE sends PDU Session Establishment message to add new non-3GPP access leg to the existing MA PDU Session. After the access leg over the new non-3GPP access is established, the UE and UPF starts sending traffic over the new non-3GPP access leg and stops sending traffic over the old non-3GPP access leg. The AMF triggers AN release procedure over the old non-3GPP access and performs UDM Registration.

### 6.8.3	Procedures

This procedure assumes that the UE is registered over untrusted non-3GPP access first and then switches trusted non-3GPP access.

Figure 6.8.2-1: Overall non-3GPP access switching procedure

1.	The UE is registered over 3GPP access and untrusted non-3GPP access and established MA PDU Session. During the MA PDU Session Establishment, the UE indicates whether it supports non-3GPP access switching in the PDU Session Establishment Request message. The AMF also indicates whether it supports non-3GPP access switching to the SMF. Considering the received capabilities of the UE, AMF and SMF capability, the SMF indicates whether non-3GPP access switching is supported to the UE in the PDU Session Establishment Accept message.

2.	The UE registers over trusted non-3GPP access with new Registration type = non-3GPP access switching. The UE includes in the List Of PDU Sessions To Be Activated the MA PDU Sessions that user plane resources are established over untrusted non-3GPP access.

NOTE:	It is assumed that the selected TNGF supports the S-NSSAI of the established MA PDU Session. This could be ensured by the UE or done by the network. In case the AMF needs to redirect the UE to the other TNGF, the AMF does not trigger non-3GPP access switching and wait until redirection procedure is finished (e.g. may be done between step 3 and step 4 or UE may need to trigger registration with new TNGF and then performs non-3GPP access switching depending solutions). The details of redirection is in the scope of 5WWC_Ph2 study.

3.	The AMF may perform authentication procedure based on existing procedure.

4.	The AMF sends Initial UE Context Setup Request message to the TNGF.

5.	After the Initial UE Context Setup procedure is completed, the AMF notifies to the SMF that the UE requested non-3GPP access switching.

6-7.	The SMF establishes user plane resources over the trusted non-3GPP access. At this point, there can be three user plane tunnels (i.e. 3GPP access, untrusted non-3GPP access, trusted non-3GPP access) in the UE and UPF. When the SMF establishes user plane resources over the trusted non-3GPP access, the SMF shall set the target access type to "trusted non-3GPP access" so that the AMF delivers the N2 information to the trusted non-3GPP access.

8.	The AMF performs UDM Registration by triggering Nudm_UECM_Registration service operation. The UDM triggers Nudm_UECM_DeregistrationNotify service according to the existing procedure. After this point, all signalling and Reachability procedures are performed over the trusted non-3GPP access.

9.	The AMF performs AN release procedure over the untrusted non-3GPP access. As a result of this procedure, user plane resources over untrusted non-3GPP access of MA PDU is completely released. When the UE and UPF recognize that user plane resources over untrusted non-3GPP access are released, the UE and UPF starts to send traffic over trusted non-3GPP access.

10.	The AMF sends Registration Accept message to the UE. When the UE receives Registration Accept message, the UE considers that the UE is deregistered from untrusted non-3GPP access and registered over trusted non-3GPP access.

6.8.4	Impacts on Existing Nodes and Functionality

UE:

-	Performs Registration with new registration type.

-	Indicates to the SMF that the UE supports non-3GPP access switching during the MA PDU Session Establishment.

-	Temporarily maintains simultaneous parallel user plane tunnel over untrusted 3GPP access and trusted non-3GPP access.

-	When the UE receives the Registration Accept message for the target non-3GPP access, the UE considers that it is deregistered from the source non-3GPP access.

AMF:

-	During the Registration procedure, delay UDM registration until non-3GPP access switching is completed.

-	Indicates to the SMF that the AMF supports non-3GPP access switching during the MA PDU Session Establishment.

-	Notifies to the SMF that the UE requested non-3GPP access switching during the Registration procedure.

-	Performs AN release over old access during the Registration procedure

SMF:

-	Indicates to the UE whether the MA PDU Session supports non-3GPP access switching during the MA PDU Session Establishment.

UPF:

-	Temporarily maintains simultaneous parallel user plane tunnel over untrusted 3GPP access and trusted non-3GPP access.

## 6.9	Solution #5.3: Path switching between non-3GPP accesses

### 6.9.1	Introduction

The solution in clause 6.9 addresses the objective of KI#5, i.e. it specifies how the data traffic of an MA PDU Session to be switched between two non-3GPP access paths, both of them using the same PLMN.

### 6.9.2	High-level Description

This solution uses the following principles to switch the data traffic of an MA PDU Session between two Non-3GPP access paths:

a.	UE uses existing procedures to register to 5GC via a Non-3GPP access network (e.g. an untrusted Non-3GPP access network). Optionally, the UE may also register to 5GC via a 3GPP access network (e.g. via NG-RAN).

b.	MA PDU Session is established using the existing procedures specified in clause 4.22.2 of TS 23.502 [3] and data traffic is transferred over the paths of the MA PDU Session.

c.	UE discovers a new Non-3GPP access network (e.g. a trusted Non-3GPP access networks) and registers to 5GC via this access network by indicating "Non-3GPP path switch". This allows the AMF to maintain temporarily two registrations via two non-3GPP accesses until the path switch is completed.

d.	UE initiates the path switch by sending, via the new Non-3GPP access network, an MA PDU Session Establishment Request (using the same PDU session ID), which triggers the SMF to establish user-plane resources for the MA PDU Session over the new Non-3GPP access network.

e.	When the user-plane resources for the MA PDU Session over the new Non-3GPP access network are established, the SMF triggers the AMF to deregister the UE from the old Non-3GPP access network (e.g. the untrusted Non-3GPP access network).

f.	The UE can trigger the path switching procedure based on internal implementation Alternatively, the UE can decide to switch data traffic based on URSP policies (if available) received from the PCF indicating the priority of a non-3GPP access (priority of the trusted and untrusted non-3GPP accesses) which are part of a MA PDU session.

### 6.9.3	Procedures

Figure 6.9.3-1 below depicts the key steps of the solution, which enables the data traffic of an MA PDU Session to be switched from a non-3GPP access path using a N3IWF to a non-3GPP access path using a TNGF. The same steps can be used to switch the data traffic of an MA PDU Session between any non-3GPP access paths (using either N3IWF or TNGF).

Figure 6.9.3-1: Procedure for enabling data traffic switching between two non-3GPP access paths

1.	[Optionally] the UE performs an initial 5G registration over 3GPP access in a PLMN. The selected AMF registers with UDM for 3GPP access and provides its GUAMI and RAT type = NR.

2.	The UE selects an N3IWF and performs an initial 5G registration over untrusted non-3GPP access in the same PLMN using step 1 to step 13 in clause 4.22.9 of TS 23.502 [3] and with the following differences and clarifications:

i)	The UE indicates its support for "Non-3GPP path switch":

-	"Non-3GPP path switch" indication indicates that this UE is capable of switching traffic of an MA PDU Session between two non-3GPP access paths.

ii)	The same AMF is selected, as the one in the previous step.

iii)	The AMF indicates to the UE its support for "Non-3GPP path switch".

iv)	The AMF registers with UDM for non-3GPP access and provides its GUAMI and RAT type = Virtual or WLAN.

NOTE:	The above procedure may be performed at any time the UE initiates a registration request (i.e. to any AN).

3.	The UE requests an MA PDU Session, as specified in clause 4.22.2 of TS 23.502 [3]. User-plane resource are established over 3GPP access and over untrusted non-3GPP access. Data traffic over the MA PDU Session is exchanged between the UE and UPF using the two accesses.

4.	The UE detects a trusted non-3GPP access network that supports 5G connectivity to the same PLMN. The UE decides to switch the data traffic transferred over the untrusted non-3GPP access of the MA PDU Session to the detected trusted non-3GPP access network as described in clause 6.9.2f. For this purpose, the UE initiates a 5G registration over trusted non-3GPP access using step 1 to step 15 in clause 4.12a.2.2 of TS 23.502 [3] with the following differences and clarifications:

i)	Indicates "Non-3GPP path switch".

-	This indication indicates that the registration over trusted non-3GPP access is required for switching the data traffic of an MA PDU Session from one non-3GPP access path to another non-3GPP access path.

ii)	After the UE is registered via trusted non-3GPP access with the indication "Non-3GPP patch switch", the AMF does not release the existing registration via untrusted non-3GPP access.

iii)	A timer value indicating how long the network will maintain two registrations via non-3GPP access is configured in the AMF. The AMF shall start a timer and maintain the two registrations via non-3GPP access until the path switch is completed or until this timer is equal to the configured timer value:

-	As shown below (see step 9), when the timer expires, the AMF initiates deregistration via the TNGF.

5.	The UE initiates the non-3GPP path switch by requesting user-plane resources via trusted non-3GPP access. To request these user-plane resources:

i)	The UE sends a PDU Session Establishment Request via the TNGF, which contains request type = MA PDU Request and the identity of the existing MA PDU Session.

ii)	The AMF sends an Update SM Context Request to SMF, which contains a "Non-3GPP path switch indication":

-	This indication informs the SMF that the PDU Session Establishment Request is sent to enable path switching from the existing non-3GPP access of the MA PDU Session to a new non-3GPP access.

iii)	The AMF inserts the "Non-3GPP path switch indication" in the Update SM Context Request because the UE is registered with type "Non-3GPP path switch" (see step 4).

iv)	The SMF initiates the user-plane resources establishment over trusted non-3GPP access.

6.	After user-plane resources over trusted non-3GPP access are established, the UE switches all uplink MA PDU Session traffic from untrusted non-3GPP access to trusted non-3GPP access. Similarly, the UPF switches all downlink MA PDU Session traffic from untrusted non-3GPP access to trusted non-3GPP access (for this the SMF modifies the existing N4 connection).

7.	The SMF sends an SM Context Status Notify message to AMF to indicate that the non-3GPP path switch has been completed.

8.	After receiving the SM Context Status Notify message, the AMF updates the UDM with RAT type = Trusted_WLAN for non-3GPP access. The AMF initiates deregistration procedure for untrusted Non-3GPP access (see clause 4.12.3 of TS 23.502 [3]). Also, the AMF stops the timer that was started in step 4. After this step, the UE has only one 5G registration via non-3GPP access.

9.	[Conditional] If AMF timer started in step 4 expires, the AMF deregisters the UE via trusted non-3GPP access. After this step, the UE has only one 5G registration via non-3GPP access.

### 6.9.4	Impacts on Existing Nodes and Functionality

UE:

-	Indicates to the AMF that the UE supports non-3GPP access switching during the Registration Request message

-	Trigger the switching from one N3GPP path to another N3GPP path is based on UE implementation.

-	Use a new indication when performing the switching during the registration procedure via the other Non-3GPP path.

AMF:

-	Indicate to SMF that Non-3GPP path switching is required for the MA PDU session.

-	A configured timer value indicating how long the network will maintain two registrations via non-3GPP accesses.

-	Configured timer value used to trigger a de-registration if a non-3GPP access path switch has not completed within the allotted provisioned time.

SMF/UPF:

-	Perform the traffic switching from untrusted N3GPP access to trusted N3GPP access or vice versa.

UDM:

-	UDM is only expected to be updated with the new RAT type information (e.g. from Virtual/WLAN to Trusted_WLAN) after the path switching has been performed successfully.

## 6.10	Solution #5.4: Non-3GPP access path switching in MA PDU Session

### 6.10.1	Introduction

This solution addresses KI#5 on switching traffic of an MA PDU Session between two non-3GPP access paths.

### 6.10.2	High-level Description

In this solution, it is assumed that the UE is able to register to 5GC over two non-3GPP access paths for the duration of switching the non-3gpp access path of an established MA PDU Session, and the duration is decided by the network. Each non-3GPP access path is identified by the RAT Type of the non-3GPP access network (e.g. one non-3GPP access path is using an N3IWF, and the other non-3GPP access path is using an TNGF).

It is assumed that the Non-3GPP access path switching feature is supported only for the MA PDU Session, therefore any UE which does not have ATSSS Capability cannot request to the network of two Non-3GPP access registrations for purpose of Non-3GPP access path switching of an MA PDU Session.

The key points of the solution are as follows:

-	The UE triggers the non-3GPP access path switching of the established MA PDU Session. The AMF detects this by presence of ATSSS path switching indication, and List of PDU Sessions To Be Activated included in the Registration request.

-	The AMF determines the RAT Type of the target non-3GPP access path which the Registration request is sent, and the RAT Type of the source non-3GPP access path which the UE is already registered over non-3GPP access. The AMF provides both source RAT Type and target RAT Type to the SMF, and requests to SMF the Access Path Switching Lifetime Value. The Access Path Switching Lifetime Value is used to decide the duration of switching. The AMF decides the value of Access Path Switching Timer based on its local configuration and the value received from the SMF, and provides to the UE.

-	The UE requests MA PDU Session Establishment via target non-3GPP access network before the Access Path Switching Timer is expired. The AMF determines whether this request is for the second non-3GPP access path, and requests to the SMF to allocate N3 CN Tunnel Info for Target Access Network. The SMF distinguishes two non-3GPP access paths by Target RAT Type and Source RAT Type provided by the AMF.

-	The target non-3GPP access network also allocates the N3 AN Tunnel Info. If the AMF receives the successful result in N2 Session Response and the SMF receives the AN Tunnel Info of  target non-3GPP access network, the access traffic switching is possible, and the traffic is transmitted via the target non-3GPP access network.

-	The SMF initiates the MA PDU Session release over the source non-3GPP access. Any corresponding SM Context and UE Context is removed from the network. UP resources via source non-3GPP access are released.

-	The AMF initiates the Deregistration of the UE over the source non-3GPP access.

### 6.10.3	MA PDU Session Access Path Switching procedure

Figure 6.10.3-1: Non-3GPP access path switching procedure for an MA PDU Session

The above Figure 6.10.3-1 shows how the non-3GPP access path of an MA PDU Session is switched to another Non-3GPP access path.

0.	The UE is registered over untrusted non-3GPP access and established MA PDU Session. The UE may be registered over 3GPP access

Editor's note:	Whether and how the UE can be aware that the network supports non-3GPP access path switching for MA PDU Session is FFS.

1.	The UE determines that the existing non-3GPP access path of the MA PDU Session needs to be changed to trusted non-3GPP access.

2.	In the Registration Request message, the UE includes ATSSS path switching indication and List Of PDU Sessions To Be Activated. ATSSS path switching indication indicates that this request is for the registration for the second non-3GPP access path via target non-3GPP access network (the trusted non-3GPP access in this case). List Of PDU Sessions To Be Activated includes the PDU Session ID of the MA PDU Session.

The same AMF is selected using GUAMI provided by the UE. If the AMF supports non-3GPP access path switching for MA PDU Session, the AMF determines whether this request is for registration over the second non-3GPP access path of the established MA PDU Session. AMF determines RAT Type of trusted non-3GPP access (as defined in clause 5.3.2.3 of TS 23.501 [2]), and checks if it is different with that of the existing non-3GPP access path of the MA PDU Session.

3.	The AMF verifies whether the ATSSS path switching is supported for the UE from the subscription data.

Editor's note:	If the AMF can verify whether the UE is allowed to register over two non-3GPP accesses for non-3GPP access path switching for the MA PDU Session based on capability information provided by the UE, instead of checking subscription data, is FFS.

4.	The AMF invokes the Nsmf_PDUSession_UpdateSMContext Request which includes the ATSSS path switching indication, PDU Session ID of the MA PDU Session, Target RAT Type (e.g. trusted W-LAN), and Source RAT Type (e.g. untrusted non-3GPP).

5.	The SMF sends the Nsmf_PDUSession_UpdateSMContext Response which includes Access Path Switching Lifetime Value. The Access Path Switching Lifetime Value indicates how long the SMF will accept the MA PDU Session Establishment request over the second non-3GPP access path. The SMF starts a timer corresponding to the Access Path Switching Lifetime Value, and if the SMF does not receive any successful Nsmf_PDUSession_UpdateSMContext Request including AN Tunnel Info over the target non-3GPP access before expiry of this timer, the SMF maintains the source non-3GPP access path.

Editor's note:	It is FFS whether the Access Path Switching Lifetime is provided by SMF to AMF or whether it is determined by the AMF itself. When determined by AMF, the Access Path Switching Lifetime indicates how long the AMF can maintain two simultaneous registrations over non-3GPP access for the UE.

6.	The AMF sends Registration Accept which includes Access Path Switching Timer. The AMF starts De-registration timer corresponding to the Access Path Switching Timer. The AMF can decide Access Path Switching Timer as longer value than Access Path Switching Lifetime Value received from the SMF in step 5, to avoid triggering the de-registration before the MA PDU Session Establishment over the second non-3GPP access path is finished.

7.	The UE requests MA PDU Session Establishment over the trusted non-3GPP access before the expiry of the Access Path Switching Timer received in step 6.

8.	The AMF invokes the Nsmf_PDUSessin_CreateSMContext Request which includes ATSSS path switching indication, PDU Session ID of the MA PDU Session, Target RAT Type, and Source RAT Type. The SMF detects that this MA PDU Session request is related to the request of step 4.

9.	The CN Tunnel Info for trusted non-3GPP access is allocated.

10.	The SMF sends Namf_Communication_N1N2MessageTransfer which includes CN Tunnel Info for trusted non-3GPP access in N2 SM information.

11.	The AMF sends N2 Session Request to the TNGF.

12.	The TNGF issues AN specific resource setup with the UE, and the the AN Tunnel Info for the trusted non-3GPP access is allocated.

13.	The Target Access Network sends N2 Session Response which includes the AN Tunnel Info corresponds to the TNGF address of the N3 tunnel for the MA PDU Session.

14.	The AMF sends the N2 SM information received from TNGF and the RAT Type of trusted non-3GPP access to the SMF.

15.	The SMF initiates an N4 Session modification procedure with the UPF. The SMF provides the AN Tunnel Info to the UPF. The UPF provides an N4 Session Modification Response to the SMF. After this step, there can be three user plane tunnels (i.e. 3GPP access, untrusted non-3GPP access, and trusted non-3GPP access) in the UE and the UPF. The UE switches all uplink traffic from untrusted non-3GPP access to trusted non-3GPP access and the UPF switches all downlink traffic from untrusted non-3GPP access to trusted non-3GPP access.

Editor's note:	It is FFS whether the UE can switch the uplink traffic from untrusted non-3GPP access to trusted non-3GPP access before step 15, and whether the existing procedure in TS 23.502 [3] can be reused.

16.	The SMF initiates the MA PDU Session release over untrusted non-3GPP access. Any corresponding SM Context and UE Context is removed from the network. UP resources over untrusted non-3GPP access is deactivated.

17.	The AMF initiates the Deregistration of the UE over untrusted non-3GPP access.

### 6.10.4	Impacts on Existing Nodes and Functionality

UE:

-	Triggers the non-3GPP access path switching of MA PDU Session.

-	Indicates in Registration request that this request is for the registration for the second non-3GPP access path by using ATSSS path switching indication and the List Of PDU Session To Be Activated.

Editor's note:	Whether and how the UE can be aware that the network supports non-3GPP access path switching for MA PDU Session is FFS.

AMF:

-	Verifies whether the ATSSS path switching is supported for the UE from the subscription data.

Editor's note:	If the AMF can verify whether the UE is allowed to register two non-3GPP accesses for non-3GPP access path switching for MA PDU Session based on capability information provided by the UE instead of checking subscription data is FFS.

-	Determines whether the Registration request from the UE is for registration over the second non-3GPP access path of the established MA PDU Session by using ATSSS path switching indication and the List Of PDU Session To Be Activated

-	Provides to SMF the RAT Type of the target non-3GPP access and the RAT Type of the source non-3GPP access.

-	Decides the Access Path Switching Timer and provides to the UE to inform the duration of non-3GPP access path switching.

SMF:

-	Decides the Access Path Switching Lifetime Value and provides to the AMF to assist the AMF to finally decide the value of Access Path Switching Timer for the UE.

UPF:

-	Allocates the N3 CN Tunnel Info for the second non-3GPP access path.

## 6.11	Solution #2.2: MPQUIC steering functionality using UDP proxying over HTTP

### 6.11.1	Introduction

The solution in clause 6.11 specifies a new ATSSS steering functionality, called Multipath QUIC (MPQUIC) steering functionality, and addresses the objective of KI#2 for a QUIC-based steering functionality. It borrows several aspects from solutions studied in TR 23.700-93 [5], mainly from Solution #6 "MPQUIC-LL Steering Functionality" and from Solution #14 "Proxy based solution using MP-QUIC".

The solution is primarily based on the draft-ietf-masque-connect-udp [27], which specifies how UDP traffic can be transferred between a client (UE) and a proxy (UPF) using the HTTP/3 protocol [28]. The HTTP/3 protocol operates on top of the QUIC protocol [6], which supports simultaneous communication over multiple paths, as defined in draft-ietf-quic-multipath [10].

The solution considers the following modes for transmitting a UDP flow (see further details in clause 6.11.3, step 5):

-	Datagram mode 1: This mode encapsulates the UDP data into QUIC DATAGRAM frames. It provides unreliable transport and adds sequence numbers to the transmitted UDP data, so that the received UDP data can be re-ordered, and the duplicated UDP data can be removed.

-	Datagram mode 2: This mode encapsulates the UDP data into QUIC DATAGRAM frames. It provides unreliable transport but does not add sequence numbers to the transmitted UDP data. Therefore, it may result in UDP data delivery with out-of-order packets and/or with duplicated packets.

-	Stream mode: This mode encapsulates the UDP data into QUIC STREAM frames. It provides reliable transport (based on the existing mechanisms supported by the QUIC protocol) and supports data delivery without out-of-order packets and without duplicated packets.

NOTE:	Stream mode provides strict reliability and in-order delivery with re-transmissions and therefore leads to melt down phenomena [see http://sites.inka.de/~W1011/devel/tcp-tcp.html] when reliable (e.g. QUIC) is carried, or counteracts application decisions when UDP is selected to avoid reliability and/or in-order delivery.

The Datagram mode 1 and the Stream mode are particularly useful for supporting per-packet splitting, i.e. when the packets of a UDP flow are split across multiple accesses and may be received out-of-order. The Datagram mode 2 is a simple transport mode which can be applied for applications that can tolerate packet reordering or duplicated packets.

Editor's note: It is FFS whether all or some (and which) of the above transport modes are needed. Further analysis is needed to determine the final set of supported transport modes.

### 6.11.2	High-level Description

The key principles of the solution are summarized below.

-	After the MA PDU Session establishment, the UE creates one or more multipath QUIC connections with the UPF. Each multipath QUIC connection is associated with a QoS flow, i.e. it carries the traffic mapped to a QoS flow.

-	The UE operates as a connect-udp client and the UPF operates as a connect-udp proxy, both defined in draft-ietf-masque-connect-udp [27]. Therefore, the UE supports an HTTP/3 client and the UPF supports an HTTP/3 proxy, both of them operating over QUIC.

-	When the UE wants to transmit the first uplink packet of a new UDP flow, the UE:

-	Selects which QUIC connection will be used for the uplink traffic of the UDP flow based on the QoS flow associated with the UDP flow;

-	Creates a new bidirectional QUIC stream on the selected QUIC connection;

-	Configures the QUIC stream to apply a steering mode (i.e. the steering mode that should be used for the uplink traffic of the UDP flow based on the ATSSS rules);

-	Sends to UPF, via the QUIC stream, an extended HTTP CONNECT request [27] containing 'path' information that identifies a destination address and port for the UDP flow (i.e. identifies the remote host where the UDP flow should be forwarded to); and

-	Forwards to UPF the uplink packets of the UDP flow using multipath QUIC transport.

-	When the UPF wants to transmit a downlink packet of a UDP flow, the UPF:

-	Selects which QUIC connection will be used for the downlink traffic of the UDP flow based on the QoS flow associated with the UDP flow (this QUIC connection is the same as the one selected by UE for the UDP flow, assuming the QoS flow in UL and DL directions is the same);

-	Selects a bidirectional QUIC stream on the selected QUIC connection (this QUIC stream is the same as the one created by the UE for the UDP flow);

-	Configures the QUIC stream to apply a steering mode (i.e. the steering mode that should be used for the downlink traffic of the UDP flow based on the N4 rules); and

-	Forwards to UE the downlink packets of the UDP flow using multipath QUIC transport.

The following figure illustrates how the traffic of a UDP flow is transferred between the UE and UPF using multipath QUIC transport [9]. In this figure, it is assumed that the uplink traffic and the downlink traffic of the UDP flow are mapped to the same QoS flow (QFI-1). Therefore, both the uplink traffic and the downlink traffic of the UDP flow use the same multipath QUIC connection.

Figure 6.11.2-1: Using multipath QUIC transport for a UDP flow

The bidirectional QUIC stream is established by the UE to enable transmission of uplink data packets (blue) and downlink data packets (red) of the UDP flow. The UE configures this stream to send uplink traffic with a steering mode determined based on the ATSSS rules in the UE (uplink steering mode). The UPF configures this stream to send downlink traffic with a steering mode determined based on the N4 rules in the UPF (downlink steering mode). The data packets of the UDP flow shown in Figure 6.11.2-1 are transmitted in datagram mode (mode 1 or mode 2), i.e. they are encapsulated in HTTP datagrams and QUIC DATAGRAM frames, each one carrying header information (see the Quarter Stream ID defined in draft-ietf-masque-h3-datagram [29]) that associates it with the established bidirectional stream. As discussed below, the data packets of the UDP flow may be transmitted in stream mode (instead of datagram mode), i.e. transmitted directly over the bidirectional stream. In this case, the data packets of the UDP flow are encapsulated in DATAGRAM capsules (see [29]) and QUIC STREAM frames.

Figure 6.11.2-2 and Figure 6.11.2-3 illustrate the components of the MPQUIC steering functionality used to support data transmission in the uplink and downlink direction respectively. The MPQUIC steering functionality is composed of three components:

1)	QoS flow selection & Steering mode selection: This component in the UE initiates the establishment of one or more QUIC connections, after the establishment of the MA PDU Session and, for each uplink UDP flow, it selects a QoS flow (based on the QoS rules), a steering mode (based on the ATSSS rules) and a transport mode (see further details below). This component in the UPF selects, for each downlink UDP flow, a QoS flow (based on the N4 rules), a steering mode (based on the N4 rules) and a transport mode (see further details below).

In the UE, this component is only used in the uplink direction, while, in the UPF, this component is only used in the downlink direction.

2)	HTTP/3 layer: Supports the HTTP/3 protocol defined in draft-ietf-quic-http [28] and the extensions defined in:

-	draft-ietf-masque-connect-udp [27] for supporting UDP proxying over HTTP;

-	draft-ietf-masque-h3-datagram [29] for supporting HTTP datagrams; and

-	draft-ietf-httpbis-h3-websockets [30] for supporting Extended CONNECT.

The HTTP/3 layer selects a QUIC connection to be used for each UDP flow and allocates a new QUIC stream on this connection that is associated with the UDP flow. It also configures this QUIC stream to apply a specific steering mode.

In the UE, the HTTP/3 layer implements an HTTP/3 client, while, in the UPF, it implements an HTTP/3 proxy.

3)	QUIC layer: Supports the QUIC protocol as defined in the applicable IETF specifications (RFC 9000 [6], RFC 9001 [7], RFC 9002 [8]) and the extensions defined in:

-	RFC 9221 [9] for supporting unreliable datagram transport with QUIC; and

-	draft-ietf-quic-multipath [10] for supporting QUIC connections using multiple paths simultaneously.

Figure 6.11.2-2: Components of MPQUIC steering functionality used for UL data transmission

Figure 6.11.2-3: Components of MPQUIC steering functionality used for DL data transmission

The protocol stack of the solution is depicted in Figure 6.11.2-4 below.

Figure 6.11.2-4: UP protocol stack of the solution

### 6.11.3	Procedures

Figure 6.11.3-1 below depicts the key steps of the procedure that enables data traffic to be exchanged between the UE and UPF using the MPQUIC steering functionality.

Figure 6.11.3-1: Procedure for enabling data traffic using the MPQUIC steering functionality

1.	The UE establishes a MA PDU Session with the 5G core (5GC) network. During the MA PDU Session establishment:

-	In the PDU Establishment Request message, the UE indicates that it supports the MPQUIC steering functionality. This indication can be used by the network (a) to select a UPF that supports the MPQUIC steering functionality and (b) to decide whether the ATSSS/N4 rules for the MA PDU Session may use the MPQUIC steering functionality.

-	The UE receives MPQUIC proxy information, i.e. one IP address of UPF, one UDP port number and the proxy type (e.g. "connect-udp"). This information is used by the UE for establishing QUIC connections with the UPF, which is also referred to as "MPQUIC proxy".

-	The UE receives one IP address/prefix for the MA PDU Session and two additional IP addresses/prefixes, called "link-specific multipath QUIC" addresses; one associated with 3GPP access and another associated with non-3GPP access. These two addresses can be used by the UE to create two paths in a multipath QUIC connection.

-	The UE receives QoS rules and ATSSS rules to be applied for the MA PDU Session, for QoS enforcement and traffic steering enforcement respectively. Similar rules (N4 rules) are received by UPF.

2.	After the MA PDU Session is established and the UE identifies that one or more ATSSS rules require traffic steering using the MPQUIC steering functionality, the UE determines the number of multipath QUIC connections to be established with the UPF (MPQUIC proxy). For example, the UE may determine to establish as many multipath QUIC connections, as the number of QoS flows of the MA PDU Session, i.e. one multipath QUIC connection per QoS flow. The QoS rules provided to UE include downlink QoS information and the UE applies the downlink QoS information to establish QUIC connections for the QoS flows used for downlink traffic only.

3.	The UE establishes the number of multipath QUIC connection with the UPF (MPQUIC proxy) determined in the previous step. This results into several multipath QUIC connections between the UE and UPF, each one composed of multiple paths, e.g. one path over 3GPP access and another path over non-3GPP access.

NOTE 1:	Data transmitted over a multipath QUIC connection must be encrypted according to RFC 9001 [7]. However, encryption might not be necessary when the multipath QUIC connection is established between UE and UPF, because the underlying 5G security mechanisms can be applied.

Editor's note: Whether and how encryption in the QUIC layer can be omitted is FFS and need to be verified by SA WG3. The impact due to encryption regarding overhead and performance is FFS.

During a QUIC connection establishment, the UE and UPF negotiate QUIC transport parameters and indicate (a) support of QUIC Datagram frames and (b) support of multipath. They indicate support of QUIC Datagram frames by providing the "max_datagram_frame_size" transport parameter with a non-zero value (see RFC 9221 [9]) and they indicate support of multipath by providing the "enable_multipath" transport parameter (see draft-ietf-quic-multipath [10]).

After a QUIC connection establishment, the HTTP/3 client and the HTTP/3 proxy negotiate HTTP settings and indicate support of HTTP Datagrams (see draft-ietf-masque-h3-datagram [29]) and support of Extended CONNECT (see draft-ietf-httpbis-h3-websockets [30]).

The QoS flow associated with a QUIC connection is also negotiated between the UE and UPF. This is done by using a new QUIC transport parameter (defined by 3GPP) when the QUIC connection is established.

NOTE 2: QUIC transport parameter needs to be registered in IANA (by stage 3).

4.	An app in the UE generates a new data packet (also referred to as "UDP payload") that should be sent via the MA PDU Session. This data packet initiates a new UDP flow, i.e. a sequence of data packets using the same 5-tuple. In the example shown in Figure 6.11.3-1, the data packet should be sent to IP address 144.23.1.47 and to UDP port 556.

5.	For the new UDP flow (and for each new UDP flow):

-	The UE selects a QoS flow (QFI) over which the UDP flow should be transmitted. This is selected by using the received QoS rules.

-	The UE selects a steering mode that should be applied for the UDP flow. This is selected by using the received ATSSS rules.

-	The UE selects a transport mode, e.g. a datagram transport mode or the stream transport mode. This is selected by using the received ATSSS rules, i.e. each ATSSS rule which indicates that the MPQUIC steering functionality should be applied for the matching traffic, indicates also the transport mode that should be applied for this traffic.

The datagram transport mode supports the following two sub-modes of operation:

-	Datagram mode 1 (with sequence numbers): The HTTP/3 proxy/client prefix each UDP data with a sequence number before passing it to the QUIC layer for multipath transmission. The sequence numbers are applied by the receiving endpoint (HTTP/3 client/proxy) to re-order the UDP data and remove duplicated UDP data.

Editor's note:	It is FFS how to ensure re-ordering on the receiver side is implemented and activated, when needed.

Editor's note:	It is FFS how same re-ordering in both Down- and Uplink can be provided which is pre-requisite for a consistent and good user experience.

Editor's note:	http3 does not specify sequencing. It is FFS which IETF sequencing scheme can/will be used.

-	Datagram mode 2 (without sequence numbers): The HTTP/3 proxy/client does not prefix each UDP data with a sequence number before passing it to the QUIC layer for multipath transmission. This may result (depending on the applied steering mode) in data delivery with out-of-order packets and/or with duplicated packets. For some applications, however, such type of data delivery may be acceptable.

Editor's note:	It is FFS whether this Datagram mode 2 is needed, when the same can be achieved by Datagram mode 1 without applying re-ordering.

Editor's note:	It is FFS whether this Datagram mode 2 is needed as ATSSS-LL can provide same functionality with much less overhead when the only advantage is packet-loss detection.

In both datagram modes, every UDP data is encapsulated within an HTTP DATAGRAM, which if further encapsulated within an QUIC DATAGRAM frame [9]. The payload of the HTTP DATAGRAM is composed of a Context ID and a Payload (as defined in draft-ietf-masque-connect-udp [27]): HTTP DATAGRAM payload = {Context ID (i), Payload (..)}.

The Datagram mode 1 and Datagram mode 2 use two different (and pre-defined in 3GPP) Context IDs, e.g. Context ID=0 and Context ID=1, respectively. With Context ID=0, the Payload contains the UDP data, whereas, with Context ID=1, the Payload contains a sequence number followed by the UDP data. The format of QUIC DATAGRAMs used in both datagram modes is shown below.

| Datagram mode 1 |  |
| --- | --- |
| Datagram mode 2 |  |

Editor's note:	It is FFS how the MPQUIC steering functionality does re-ordering for the Datagram mode 1 (with sequence numbers). It is FFS whether and which parts of re-ordering mechanisms can be specified by IETF, should be specified by 3GPP and can be left for implementation.

Editor's note:	It is FFS whether Stream mode and/or Datagram mode 2 are needed. Both do not support required rel18 functionality for non-TCP traffic splitting of unreliable traffic, including the transport mode negotiation.

6.	The UE selects a multipath QUIC connection to be used for the new UDP flow (e.g. based on the selected QFI) and the UE allocates a new bidirectional QUIC stream (e.g. stream 40) in this multipath QUIC connection. This new stream is associated with the new UDP flow. The UE configures the new stream to transmit uplink data traffic using the selected steering mode for this UDP flow.

7-9.	The UE sends the data packet using the allocated new stream on the selected QUIC connection.

When the datagram transport mode is selected (either mode 1 or mode 2), the UE encapsulates the data packet within an HTTP DATAGRAM frame [27], which is transferred inside a QUIC DATAGRAM frame [9]. The header of the HTTP DATAGRAM indicates that this datagram is associated with stream 40 (i.e. the Quarter Stream ID is set to 10).

When the stream transport mode is selected, the UE encapsulates the data packet within an HTTP DATAGRAM frame [29] that is further encapsulated in a DATAGRAM capsule (see clause 4.4 in [29]), which is transferred inside a QUIC STREAM frame [6].

In the example procedure shown in Figure 6.11.3-1, the datagram transport mode (either mode 1 or mode 2) is selected. The UE sends an HTTP CONNECT request via the allocated stream 40, which indicates to UPF (MPQUIC proxy) that the UE wishes to create a tunnel to send UDP traffic to remote host 144.23.1.47 and port 556. The :protocol pseudo-header is set to "connect-udp" (as defined in [27]) to indicate that the protocol to be spoken on the tunnel is the capsule protocol defined in draft-ietf-masque-h3-datagram [29]. Finally, the UE sends a QUIC DATAGRAM frame to UPF that encapsulates the data packet, which is forwarded to the remote host (144.23.1.47:556). Note that the QUIC DATAGRAM frame may contain a sequence number (as defined in step 5).

If the stream transport mode were selected, the HTTP DATA frame in step 8 would contain a DATAGRAM capsule, which would include the data packet in the HTTP Datagram Payload field (see [29]).

10.	The UPF (MPQUIC proxy) responds with an HTTP 200 status, indicating that the request to proxy data packets to a remote host 144.23.1.47 and destination port 556 is accepted.

11.	When a data packet is received by UPF (MPQUIC proxy) from the remote host (data packet #2), this data packet is transferred to the UE using the established context information for the UDP flow, i.e. using the selected multipath QUIC connection, the selected stream on this connection, the selected steering mode, and the selected transport mode. Such context information is stored in the UPF and in the UE and is applied for all the data packets of the UDP flow.

12.	Similarly, when another data packet is generated by the UE app (data packet #3), this data packet is transferred to UPF (MPQUIC proxy) using again all the stored context information for the UDP flow.

NOTE 2:	The context information for a UDP flow in the UE and in the UPF is created when the first data packet (i.e. Data packet #1) of this UDP flow is transferred. All subsequent data packets of the same UDP flow are transferred between the UE and the UPF using this context information.

When the UE identifies that the context information for a UDP flow is no longer needed (e.g. the app closes the associated UDP socket), the UE deletes this context information and releases the associated QUIC stream, which cause the UPF to delete the context information stored in UPF.

The following figure 6.11.3-2 illustrates an example of how the uplink traffic of various UDP flows is transferred from the UE to UPF using QUIC multipath transport, and how the UPF relays this traffic to a final destination (remote host). Note that, for each UDP flow, there is an associated QUIC connection and an associated bidirectional QUIC stream, which is configured to apply a specific steering mode for the uplink traffic. For example, the red UDP flow is associated with the multipath QUIC connection #1 and with the red Stream Y, which is configured to apply the Smallest delay steering mode in the uplink direction.

All UDP flows shown in this figure, except the blue UDP flow, are transferred with the datagram transport mode (mode 1 or mode 2), so their data packets are transferred inside QUIC DATAGRAM frames. The blue UDP flow is transferred with the stream transport mode, so its data packets are transferred inside QUIC STREAM frames.

The downlink traffic of UDP flows is transferred between the UE and UPF in a similar way.

Figure 6.11.3-2: Example of user-plane operation using the MPQUIC steering functionality (UL direction)

### 6.11.4	Impacts on Existing Nodes and Functionality

Editor's note:	FFS.

Editor's note:	Protocol overhead to be provided, i.e. FFS.

## 6.12	Solution #2.3: MPQUIC steering functionality using IP proxying over HTTP

### 6.12.1	Introduction

The solution in clause 6.12 specifies a new ATSSS steering functionality, called Multipath QUIC-IP (MPQUIC-IP) steering functionality, and addresses the objective of KI#2 for a QUIC-based steering functionality. It borrows several aspects from solutions studied in TR 23.700-93 [5], mainly from Solution #6 "MPQUIC-LL Steering Functionality" and from Solution #14 "Proxy based solution using MP-QUIC".

The solution is primarily based on the draft-ietf-masque-connect-ip [31], which specifies how IP traffic can be transferred between a client (UE) and a proxy (UPF) using the HTTP/3 protocol [28]. The HTTP/3 protocol operates on top of the QUIC protocol [6], which supports simultaneous communication over multiple paths, as defined in draft-ietf-quic-multipath [10].

The solution considers the following modes for transmitting an IP flow (see further details in clause 6.12.3, step 5):

-	Datagram mode 1: This mode encapsulates the IP packets into QUIC DATAGRAM frames. It provides unreliable transport and adds sequence numbers to the transmitted IP packets, so that the received IP packets can be re-ordered, and the duplicated IP packets can be removed.

-	Datagram mode 2: This mode encapsulates the IP packets into QUIC DATAGRAM frames. It provides unreliable transport but does not add sequence numbers to the transmitted IP packets. Therefore, it may result in data delivery with out-of-order packets and/or with duplicated packets.

-	Stream mode: This mode encapsulates the IP packets into QUIC STREAM frames. It provides reliable transport (based on the existing mechanisms supported by the QUIC protocol) and supports data delivery without out-of-order packets and without duplicated packets.

NOTE:	Stream mode provides strict reliability and in-order delivery with re-transmissions and therefore leads to melt down phenomena [see http://sites.inka.de/~W1011/devel/tcp-tcp.html] when reliable (e.g. QUIC) is carried, or counteracts application decisions when UDP is selected to avoid reliability and/or in-order delivery.

The Datagram mode 1 and the Stream mode are particularly useful for supporting per-packet splitting, i.e. when the packets of an IP flow are split across multiple accesses and may be received out-of-order. The Datagram mode 2 is a simple transport mode which can be applied for applications that can tolerate packet reordering or duplicated packets.

Editor's note:	It is FFS whether all or some (and which) of the above transport modes are needed. Further analysis is needed to determine the final set of supported transport modes.

### 6.12.2	High-level Description

The key principles of the solution are summarized below.

-	After the MA PDU Session establishment, the UE creates one or more multipath QUIC connections with the UPF. Each multipath QUIC connection is associated with a QoS flow, i.e. it carries the traffic mapped to a QoS flow.

-	The UE operates as a connect-ip client and the UPF operates as a connect-ip proxy, both defined in draft-ietf-masque-connect-ip [31]. Therefore, the UE supports an HTTP/3 client and the UPF supports an HTTP/3 proxy, both of them operating over QUIC.

-	On each of the established QUIC connections, the UE sends an extended HTTP CONNECT request to UPF (as defined in [31]) indicating that IP proxying over HTTP is needed.

-	When the UE wants to transmit an uplink packet of an IP flow, the UE:

-	Selects which QUIC connection will be used for the uplink traffic of the IP flow based on the QoS flow associated with the IP flow;

-	Creates a new bidirectional QUIC stream on the selected QUIC connection;

-	Configures the QUIC stream to apply a steering mode (i.e. the steering mode that should be used for the uplink traffic of the IP flow based on the ATSSS rules); and

-	Forwards to UPF the uplink packets of the IP flow using multipath QUIC transport.

-	When the UPF wants to transmit a downlink packet of an IP flow, the UPF:

-	Selects which QUIC connection will be used for the downlink traffic of the IP flow (this QUIC connection is the same as the one selected by UE for the IP flow, assuming the QoS flow in UL and DL directions is the same);

-	Selects a bidirectional QUIC stream on the selected QUIC connection (this QUIC stream is the same as the one created by the UE for the IP flow);

-	Configures the QUIC stream to apply a steering mode (i.e. the steering mode that should be used for the downlink traffic of the IP flow based on the N4 rules); and

-	Forwards to UE the downlink packets of the IP flow using multipath QUIC transport.

The following figure illustrates how the traffic of an IP flow is transferred between the UE and UPF using multipath QUIC transport [9]. In this figure, it is assumed that the uplink traffic and the downlink traffic of the IP flow are mapped to the same QoS flow (QFI-1). Therefore, both the uplink traffic and the downlink traffic of the IP flow use the same multipath QUIC connection.

Figure 6.12.2-1: Using multipath QUIC transport for a IP flow

The bidirectional QUIC stream is established by the UE to enable transmission of uplink IP packets (blue) and downlink IP packets (red) of the IP flow. The UE configures this stream to send uplink traffic with a steering mode determined based on the ATSSS rules in the UE (uplink steering mode). The UPF configures this stream to send downlink traffic with a steering mode determined based on the N4 rules in the UPF (downlink steering mode). The data packets of the IP flow shown in Fig. 6.12.2-1 are transmitted in datagram mode (mode 1 or mode 2), i.e. they are encapsulated in HTTP datagrams and QUIC DATAGRAM frames, each one carrying header information (see the Quarter Stream ID defined in draft-ietf-masque-h3-datagram [29]) that associates it with the established bidirectional stream. As discussed below, the IP packets of the IP flow may be transmitted in stream mode (instead of datagram mode), i.e. transmitted directly over the bidirectional stream. In this case, the IP packets of the IP flow are encapsulated in DATAGRAM capsules (see [29]) and QUIC STREAM frames.

Figure 6.12.2-2 and Figure 6.12.2-3 illustrate the components of the MPQUIC-IP steering functionality used to support data transmission in the uplink and downlink direction respectively. The MPQUIC-IP steering functionality is composed of three components:

1)	QoS flow selection & Steering mode selection: This component in the UE initiates the establishment of one or more QUIC connections, after the establishment of the MA PDU Session and, for each uplink IP flow, it selects a QoS flow (based on the QoS rules), a steering mode (based on the ATSSS rules) and a transport mode (see further details below). This component in the UPF selects, for each downlink IP flow, a QoS flow (based on the N4 rules), a steering mode (based on the N4 rules) and a transport mode (see further details below)

In the UE, this component is only used in the uplink direction, while, in the UPF, this component is only used in the downlink direction.

2)	HTTP/3 layer: Supports the HTTP/3 protocol defined in draft-ietf-quic-http [28] and the extensions defined in:

-	draft-ietf-masque-connect-ip [31] for supporting IP proxying over HTTP; and

-	draft-ietf-masque-h3-datagram [29] for supporting HTTP datagrams.

The HTTP/3 layer selects a QUIC connection to be used for each IP flow and allocates a new QUIC stream on this connection that is associated with the IP flow. It also configures this QUIC stream to apply a specific steering mode (further details are provided below).

In the UE, the HTTP/3 layer implements an HTTP/3 client, while, in the UPF, it implements an HTTP/3 proxy.

3)	QUIC layer: Supports the QUIC protocol as defined in the applicable IETF specifications (RFC 9000 [6],

RFC 9001 [7], RFC 9002 [8]) and the extensions defined in:

-	RFC 9221 [9] for supporting unreliable datagram transport with QUIC; and

-	draft-ietf-quic-multipath [10] for supporting QUIC connections using multiple paths simultaneously.

Figure 6.12.2-2: Components of MPQUIC-IP steering functionality used for UL data transmission

Figure 6.12.2-3: Components of MPQUIC-IP steering functionality used for DL data transmission

The protocol stack of the solution is depicted in Figure 6.12.2-4 below.

Figure 6.12.2-4: UP protocol stack of the solution

### 6.12.3	Procedures

Figure 6.12.3-1 below depicts the key steps of the procedure that enables data traffic to be exchanged between the UE and UPF using the MPQUIC-IP steering functionality.

Figure 6.12.3-1: Procedure for enabling data traffic using the MPQUIC-IP steering functionality

1.	The UE establishes a MA PDU Session with the 5G core (5GC) network. During the MA PDU Session establishment:

-	In the PDU Establishment Request message, the UE indicates that it supports the MPQUIC-IP steering functionality. This indication can be used by the network (a) to select a UPF that also supports the MPQUIC-IP steering functionality and (b) to decide whether the ATSSS/N4 rules for the MA PDU Session may use the MPQUIC-IP steering functionality.

-	The UE receives MPQUIC-IP proxy information, i.e. one IP address of UPF, one UDP port number and the proxy type (e.g. "connect-ip"). This information is used by the UE for establishing QUIC connections with the UPF, which is also referred to as "MPQUIC-IP proxy".

-	The UE receives one IP address/prefix for the MA PDU Session and two additional IP addresses/prefixes, called "link-specific multipath QUIC" addresses; one associated with 3GPP access and another associated with the non-3GPP access. These two addresses can be used by the UE to create two paths in a multipath QUIC connection.

-	The UE receives QoS rules and ATSSS rules to be applied for the MA PDU Session, for QoS enforcement and traffic steering enforcement respectively. Similar rules (N4 rules) are received by UPF.

2.	After the MA PDU Session is established and the UE identifies that one or more ATSSS rules require traffic steering using the MPQUIC-IP steering functionality, the UE determines the number of multipath QUIC connections to be established with the UPF (MPQUIC-IP proxy). For example, the UE may determine to establish as many multipath QUIC connections, as the number of QoS flows of the MA PDU Session, i.e. one multipath QUIC connection per QoS flow. The QoS rules provided to UE include downlink QoS information and the UE applies the downlink QoS information to establish QUIC connections for the QoS flows used for downlink traffic only.

3.	The UE establishes the number of multipath QUIC connection with the UPF (MPQUIC-IP proxy) determined in the previous step. This results into several multipath QUIC connections between the UE and UPF, each one composed of multiple paths, e.g. one path over 3GPP access and another path over non-3GPP access.

NOTE 1:	Data transmitted over a multipath QUIC connection must be encrypted according to RFC 9001 [7]. However, encryption might not be necessary when the multipath QUIC connection is established between UE and UPF, because the underlying 5G security mechanisms can be applied.

Editor's note:	Whether and how encryption in the QUIC layer can be omitted is FFS and need to be verified by SA3. The impact due to encryption regarding overhead and performance is FFS.

During a QUIC connection establishment, the UE and UPF negotiate QUIC transport parameters and indicate (a) support of QUIC Datagram frames and (b) support of multipath. They indicate support of QUIC Datagram frames by providing the "max_datagram_frame_size" transport parameter with a non-zero value (see RFC 9221 [9]) and they indicate support of multipath by providing the "enable_multipath" transport parameter (see draft-ietf-quic-multipath [10]).

After a QUIC connection establishment, the HTTP/3 client and the HTTP/3 proxy negotiate HTTP settings and indicate support of HTTP Datagrams (see draft-ietf-masque-h3-datagram [29]) and support of Extended CONNECT (see draft-ietf-httpbis-h3-websockets [30]).

The QoS flow associated with a QUIC connection is also negotiated between the UE and UPF. This is done by using a new QUIC transport parameter (defined by 3GPP) when the QUIC connection is established.

NOTE 2:	QUIC transport parameter needs to be registered in IANA (by stage 3).

4.	On each of the established QUIC connections, the UE sends an extended HTTP CONNECT request to UPF (as defined in [31]) indicating that IP proxying over HTTP is needed. If this is accepted by UPF, it responds with a 200 status.

5.	The UE generates a new IP packet (IP packet #1) that should be sent via the MA PDU Session. This IP packet initiates a new IP flow, i.e. a sequence of IP packets using the same 5-tuple. For this new IP flow (and for each new IP flow):

-	The UE selects a QoS flow (QFI) over which the IP flow should be transmitted. This is selected by using the received QoS rules.

-	The UE selects a steering mode that should be applied for the IP flow. This is selected by using the received ATSSS rules.

-	The UE selects a transport mode, e.g. a datagram transport mode or the stream transport mode. This is selected by using the received ATSSS rules, i.e. each ATSSS rule which indicates that the MPQUIC steering functionality should be applied for the matching traffic, indicates also the transport mode that should be applied for this traffic.

The datagram transport mode supports the following two sub-modes of operation:

-	Datagram mode 1 (with sequence numbers): The HTTP/3 proxy/client prefix each IP packet with a sequence number before passing it to the QUIC layer for multipath transmission. The sequence numbers are applied by the receiving endpoint (HTTP/3 client/proxy) to re-order the IP packets and remove duplicated packets.

Editor's note:	It is FFS how to ensure re-ordering on the receiver side is implemented and activated, when needed.

Editor's note:	It is FFS how same re-ordering in both Down- and Uplink can be provided which is pre-requisite for a consistent and good user experience.

Editor's note:	http3 does not specify sequencing. It is FFS which IETF sequencing scheme can/will be used.

-	Datagram mode 2 (without sequence numbers): The HTTP/3 proxy/client does not prefix each IP packet with a sequence number before passing it to the QUIC layer for multipath transmission. This may result (depending on the applied steering mode) in data delivery with out-of-order packets and/or with duplicated packets. For some applications, however, such type of data delivery may be acceptable.

Editor's note:	It is FFS whether this Datagram mode 2 is needed, when the same can be achieved by Datagram mode 1 without applying re-ordering.

Editor's note:	It is FFS whether this Datagram mode 2 is needed as ATSSS-LL can provide same functionality with much less overhead when the only advantage is packet-loss detection.

In both datagram modes, every IP packet is encapsulated within an HTTP DATAGRAM, which if further encapsulated within an QUIC DATAGRAM frame [9]. The payload of the HTTP DATAGRAM is composed of a Context ID and a Payload (as defined in draft-ietf-masque-connect-ip [31]): HTTP DATAGRAM payload = {Context ID (i), Payload (..)}.

The Datagram mode 1 and Datagram mode 2 use two different (and pre-defined in 3GPP) Context IDs, e.g. Context ID=0 and Context ID=1, respectively. With Context ID=0, the Payload contains the IP packet, whereas, with Context ID=1, the Payload contains a sequence number followed by the IP packet. The format of QUIC DATAGRAMs used in both datagram modes is shown below.

| Datagram mode 1 |  |
| --- | --- |
| Datagram mode 2 |  |

Editor's note:	It is FFS how the MPQUIC steering functionality does re-ordering for the Datagram mode 1 (with sequence numbers). It is FFS whether and which parts of re-ordering mechanisms can be specified by IETF, should be specified by 3GPP, and can be left for implementation.

Editor's note:	It is FFS whether Stream mode and/or Datagram mode 2 are needed. Both do not support required rel18 functionality for non-TCP traffic splitting of unreliable traffic, including the transport mode negotiation.

6.	The UE selects a multipath QUIC connection to be used for the new IP flow (e.g. based on the selected QFI) and the UE allocates a new QUIC stream (e.g. stream 40) in this multipath QUIC connection. This new stream is associated with the new IP flow. The UE configures the new stream to transmit data traffic using the selected steering mode for this IP flow.

9.	The UE sends the IP packet using the allocated new stream on the selected QUIC connection.

When the datagram transport mode is selected (mode 1 or mode 2), the UE encapsulates the IP packet within an HTTP DATAGRAM frame [29], which is transferred inside a QUIC DATAGRAM frame [9]. The header of the HTTP DATAGRAM indicates that this datagram is associated with stream 40 (i.e. the Quarter Stream ID is set to 10).

When the stream transport mode is selected, the UE encapsulates the IP packet within an HTTP DATAGRAM frame [29] that is further encapsulated in a DATAGRAM capsule (see [29]), which is transferred inside a QUIC STREAM frame [6].

In the example procedure shown in Figure 6.12.3-1, the datagram transport mode (mode 1 or mode 2) is selected. The UE sends a QUIC DATAGRAM frame to UPF that encapsulates the IP packet, which is forwarded by UPF to the remote host.

11.	When an IP packet is received by UPF (MPQUIC-IP proxy) from the remote host (IP packet #2), this IP packet is transferred to the UE using the established context information for the IP flow, i.e. using the selected multipath QUIC connection, the selected stream on this connection, the selected steering mode, and the selected transport mode. Such context information is stored in the UPF and in the UE and is applied for all the IP packets of the IP flow.

12.	Similarly, when another IP packet is generated by the UE app (IP packet #3), this IP packet is transferred to UPF (MPQUIC-IP proxy) using again all the stored context information for the IP flow.

NOTE 2:	The context information for an IP flow in the UE and in the UPF is created when the initial IP packet (i.e. IP packet #1) of this IP flow is transferred. All subsequent IP packets of the same IP flow are transferred between the UE and the UPF using this context information.

When the UE identifies that the context information for an IP flow is no longer needed, the UE deletes this context information and releases the associated QUIC stream, which cause the UPF to delete the context information stored in UPF.

The following figure 6.12.3-2 illustrates an example of how the uplink traffic of various IP flows is transferred from the UE to UPF using QUIC multipath transport, and how the UPF relays this traffic to a final destination (remote host). Note that, for each IP flow there is an associated QUIC connection and an associated bidirectional QUIC stream, which is configured to apply a specific steering mode for the uplink traffic. For example, the red IP flow is associated with the multipath QUIC connection #1 and with the red Stream Y, which is configured to apply the Smallest delay steering mode in the uplink direction.

All IP flows shown in this figure, except the blue IP flow, are transferred with the datagram transport mode (mode 1 or mode 2), i.e. their data packets are transferred inside QUIC DATAGRAM frames. The blue IP flow is transferred with the stream transport mode, so its data packets are transferred inside QUIC STREAM frames.

The downlink traffic of IP flows is transferred between the UE and UPF in a similar way.

Figure 6.12.3-2: Example of user-plane operation using the MPQUIC-IP steering functionality (UL direction)

### 6.12.4	Impacts on Existing Nodes and Functionality

Editor's note: FFS.

Editor's note:	Protocol overhead to be provided, i.e. FFS.

## 6.13	Solution #2.4: Limiting MA-PDU Per-Packet Overhead

### 6.13.1	Introduction

This solution addresses aspects of Key Issue #2, especially, the impact on the user plane performance and UE impacts.

When encapsulating IP and/or Ethernet traffic within a DCCP or QUIC tunnel, the headers of the encapsulated traffic need to be compressed to reduce the overhead per packet, using typically Ethernet Header Compression (EHC) [34, Annex A] and Robust Header Compression (ROHC) [32, 33] protocols. We can call this feature "inner header compression" (IHC). This solution describes how IHC can be established between the UE and UPF. IHC is optional and configured by the network based on network policy and UE/network capability.

### 6.13.2	High Level Description

Inner header compression can be performed by tunnel clients/servers, such as the MP-DCCP tunnel client/server defined in solution #2.1, or by the MP-DCCP or MP-QUIC layers. Using the tunnel clients/servers may be preferable since it enables using the same mechanism independently from the underlying transport protocol. Such a solution could therefore be used for any low-layer steering functionality, such as the MPDCCP-LL steering functionality defined in solution #2.1 and for the MPQUIC-IP steering functionality defined in solution #2.3. However, it is not required for high-layer steering functionalities, such as the MPQUIC steering functionality defined in solution #2.2, because no inner headers are used.

Inner header compression should be configured on both ends, i.e. UE and UPF, prior to establishing a flow. NAS signalling can be used for this purpose, as described below.

### 6.13.3	Procedures

The following procedure describes how inner header compression can be configured when a new MA PDU session is created.

Figure 6.13.3-1: Configuration of Inner Header Compression in a MA-PDU Session

In step 1, a UE application decides to initiate communication with an Application Server (AS). UE determines to use a MA PDU session. UE determines to use IHC.

In step 2, UE sends a PDU Session Establishment Request. The request includes the UE's supported IHC capabilities (e.g. the supported RoHC profiles).

In step 3, SMF determines, based on the PCC rules, which traffic of the MA PDU Session should be steered with MP-DCCP/MP-QUIC based steering functionality. SMF determines the IHC configuration based on the IHC capabilities of the UE and the network.

The same IHC configuration applies to all SDFs which are steered with a MP-DCCP/MP-QUIC based steering functionality.

In step 4, SMF sends to UPF a N4 session establishment/modification request, including an IHC configuration.

In step 5, UPF configures packet detection, enforcement and reporting rules, and configures the tunnel server to use the received IHC configuration for the appropriate MA PDU session flows.

In step 6, UPF sends a N4 session establishment response to SMF.

In step 7, SMF sends to UE, through AMF, a PDU Session Establishment Accept message including IHC configuration.

In step 8, UE configures the tunnel client to use IHC for the appropriate MA PDU session flows, based on the IHC configuration in the PDU session establishment accept message.

From this point on, the application traffic is switched/steered/split over a MA PDU session between UE and UPF. Inner headers are compressed between UE and UPF. The application traffic is forwarded by the UPF to/from the Application Server (AS).

### 6.13.4	Impacts on Existing Nodes and Functionality

IHC requires UE and UPF to implement header compression, e.g. using ROHC and/or EHC. This function may be in the tunnel client (on UE) and server (on UPF). UE indicates its IHC capabilities in the PDU session establishment request and obtains IHC configuration in PDU session establishment accept.

The SMF determines IHC configuration based on the IHC capabilities of the UE and the network. The SMF transmits the IHC configuration to UPF (over N4 signalling) and to UE (over NAS signalling).

## 6.14	Solution #3.5: Redundant Traffic Steering Mode Activation/Deactivation

### 6.14.1	Introduction

This solution addresses aspects of Key Issue #3 (Solutions for Redundant Traffic Steering Mode), especially related to the mechanism to support the activation and deactivation of traffic duplication, and the criteria needed for making this decision. In the following, we use existing solutions (MP-QUIC-LL as described in TR 23.700-93 [5], Solution #6, and MP-DCCP-LL as described in the present document, clause 6.3) as the framework to describe the solution. However, the solution is not tightly tied to these solutions, and can work for other solutions.

### 6.14.2	High Level Description

In both the MP-DCCP and MP-QUIC based solutions, the protocol stack at the UE and UPF is as shown in Figure 6.14.2.1.

Figure 6.14.2-1: Protocol Stack for MP-QUIC and MP_DCCP

The MP-QUIC/DCCP Tunnel Client (MxTC) at the UE, and the MP-QUIC/DCCP Tunnel Server (MxTS) at the UPF, rely on the underlying MP-QUIC or MP-DCCP protocol. These protocols support a number of traffic steering modes, including redundant traffic steering.

In both cases, the MxTC establishes an MP-QUIC/DCCP connection with the MxTS in the UPF. An MP-QUIC/DCCP connection carries the traffic of only one QoS flow. In addition, it is assumed that an MP-QUIC/DCCP connection will be configured with a single steering mode. The MxTC receives UL PDUs from the upper layer and, for each UL PDU, the MxTC selects a steering mode to be applied for the UL PDU based on the ATSSS rules in the UE. The MxTC subsequently selects a MP-QUIC/DCCP connection to transmit the PDU based on the selected QoS flow and the selected steering mode.

This mechanism works well for QoS flows which are known, a priori, to require redundant transmission. For such use cases, the choice of redundant transmission can be made at PDU session establishment, and traffic duplication should always be activated. In other use cases, traffic duplication should only be used when needed, as duplicating traffic over both accesses results in:

-	Power consumption at the UE: The UE is required to operate over both accesses.

-	Core network load: The redundant transmissions will result in a certain load to the 5GC.

-	Processing at UE and UPF: The UE and UPF both need to handle re-ordering and possible reception of duplicate PDUs.

-	Radio resource usage: Radio resources are used over both accesses.

For these use cases, traffic duplication should only be used when needed, and should be dynamically activated / deactivated on a per-packet basis.

NOTE:	The mechanism to duplicate the packet is FFS. It may be implemented at the MxTC or it may be implemented in the layers below the MxTC (for example in the MP-QUIC/DCCP layer).

The Activation/Deactivation decision should be made at the MxTC for the UE and at the MxTS for the UPF, and should be based on selection criteria and measurements:

-	Packet Loss Rate (PLR): This measurement indicates the rate of PDU packet loss for each access network. The measurement is applicable to both uplink and downlink transmissions.

-	Delay: This measurement indicates the PDU transmission delay for each access network. This is a one-way delay between UE and UPF for uplink transmissions, and between UPF and UE for downlink transmissions.

-	Variability of Delay: This measurement indicates the variability of PDU arrivals (jitter) over an access network. PDU arrivals at the UPF for uplink transmission and PDU arrivals at the UE for downlink transmissions. The variability of delay measurement is a reflection of transfer quality stability within certain time interval. It may be calculated using the mechanisms described in Solution #3 in TR 23.700-93 [5]. Variability of Delay measurements prevent wasteful transmissions over an access path. Some services, such as gaming and IMS voice, are sensitive to variability of delay. For these services, activating traffic duplication over a path that will not meet the variability of delay requirements, will not be useful.

-	Load: This measurement indicates the load over an access network.

The selection criteria should rely on these measurements and should allow the UE to activate traffic duplication only when "needed and effective". For example, if the PLR is of critical importance of a data flow, the duplication should be activated when the measured PLR on the active access leg(s) is bigger than the threshold. For another example, the UE should not activate traffic duplication over a non-3GPP access, if the delay over the non-3GPP access will not meet the one-way delay requirement for the PDU. In such a case, the duplicate transmissions over the non-3GPP access would be useless. The selection criteria are included in the ATSSS rules provided to the UE and the N4 Rules provided to the UPF. The measurement configuration is included in the Measurement Assistance Information.

When traffic duplication is deactivated, the UE and UPF should rely on a default steering mode (such as "Active-Standby" or "Smallest Delay") to select which access path to use. The default steering mode configuration could be chosen in a similar way how normal steering mode is configured (e.g. depending on UE/NW capabilities) and provided in the ATSSS rules to the UE and in the N4 rules to the UPF. The duplication criteria is configured in a way that the measurements in default steering mode can be used. For example, if Active-Standby is configured as the default steering mode, the PLR measurement on the active access, instead of both accesses, should be used in the criteria.

Editor's note:	How to handle out-of-order reception due to latency difference between access paths is FFS.

### 6.14.3	Procedures

The following procedure (Figure 6.14.3.1) describes how traffic duplication can be dynamically activated and/or deactivated.

Figure 6.14.3-1: Redundant Steering Mode: Activation/Deactivation of Traffic Duplication

In step 1, UE application decides to initiate communication with AS. UE determines to use a MA-PDU session.

In step 2, UE sends a PDU Session Establishment or Modification Request. The request includes capability support for redundant steering mode.

In step 3, SMF determines to use MA-PDU session, e.g. based on configuration or PCC rules. SMF determines ATSSS rules for the UE and N4 rules for UPF.

In step 4, SMF sends to UPF a N4 session establishment/modification request, including N4 rules and Measurement Assistance Information. The N4 rules include an indication whether the UPF should start with traffic duplication activated or deactivated, default steering mode configuration to use if duplication is deactivated, as well as the selection criteria to activate or deactivate traffic duplication.

In step 5, SMF sends to UE, through AMF, a PDU Session Establishment/Modification Accept message including ATSSS rules for redundant steering mode and Measurement Assistance Information. The ATSSS rules include an indication whether the UE should start with traffic duplication activated or deactivated, default steering mode configuration to use if duplication is deactivated, as well as the selection criteria to activate or deactivate traffic duplication.

In step 6, UE configures the MxTC.

In step 7, before measurements for selection criteria are available, MxTC determines whether traffic duplication should be activated or deactivated based on the indication from the PDU Session Establishment/Modification Accept message.

From this point on, the application traffic is sent over the MA-PDU session between UE and UPF. The MxTC monitors the selection criterion and based on ATSSS rules, decides whether traffic duplication should be activated or deactivated for the uplink application traffic. Similarly, the MxTS monitors the selection criterion and based on N4 rules, decides whether traffic duplication should be activated or deactivated for the downlink application traffic.

### 6.14.4	Impacts on Existing Nodes and Functionality

PCF:

-	Provide PCC rules to SMF, considering the selection criteria to activate or deactivate traffic duplication

SMF:

-	Based on the PCC rules, create ATSSS rules and N4 rules including the selection criteria to activate or deactivate traffic duplication, and default steering mode configuration to use if duplication is deactivated.

UPF:

-	Shall be able to determine whether traffic duplication is activated or deactivated.

UE:

-	Shall be able to indicate support of redundant steering mode when requesting a MA PDU Session.

-	Shall be able to determine whether traffic duplication is activated or deactivated.

## 6.15	Solution #3.6: Redundant steering mode

### 6.15.1	Introduction

This solution addresses KI#3 on support of redundant traffic steering.

### 6.15.2	High-level Description

Redundant steering mode (RSM) allows traffic to be transmitted via 3GPP and non-3GPP accesses in a redundant way to achieve the lowest latency and to lower the loss rate.

In this solution all packets are duplicated over both 3GPP and non-3GPP access when RSM applies for the traffic. This ensures that the application traffic always benefits from the increased robustness that comes with duplication of packets.

RSM can be supported when MPTCP steering functionality is used. RSM does not apply to the ATSSS-LL steering functionality.

Editor's note:	RSM for MP-QUIC/MP-DCCP is FFS

In case RSM is applied for a GBR flow, the SMF provides the related GBR QoS profile to both 3GPP and non-3GPP accesses. This ensures that the corresponding GBR resources are reserved in both accesses.

Editor's note:	It is FFS if and how AF can influence the use of RSM

### 6.15.3	Procedures

The existing procedures apply. Only a new Steering Mode is defined.

### 6.15.4	Impacts on Existing Nodes and Functionality

The PCC rules, ATSSS rules and the N4 rules are enhanced to support a new steering mode for Redundant Steering Mode.

SMF:

-	Based on the PCC rules, create ATSSS rules and N4 rules with the new Steering Mode.

PCF:

-	Provide PCC rules with the new Steering Mode.

UPF:

-	Ife RSM is used for a traffic flow, duplicate traffic via the available access paths of a MA PDU session for the downlink traffic. Handle received duplicated uplink traffic.

UE:

-	If RSM is used for a traffic flow, duplicate traffic via the available access paths of a MA PDU session for the uplink traffic. Handle received duplicated downlink traffic.

Editor's note:	It is FFS if and how AF can influence the use of RSM.

## 6.16	Solution #5.5: Non-3GPP path switch during Registration in new non-3GPP access

### 6.16.1	Introduction

### 6.16.2	High-level Description

This solution addresses Key Issue #5 "Switching traffic of an MA PDU Session between two non-3GPP access paths" by allowing a UE to Register in a new non-3GPP AN node while still keeping the UP connection in old non-3GPP AN node for a while, allowing the UE and UPF to switch the traffic to the new non-3GPP access before releasing the old access. The solution is based on UE triggering a Registration Request via the new non-3GPP AN node and requesting UP establishment using the "PDU Sessions to be activated" parameter.

The solution allows both single access PDU Sessions and MA PDU Sessions to be "switched" using the Registration procedure. A reason for this is that the solution is basically describing how a mobility Registration Update is performed via non-3GPP access. If the UE initiates a mobility Registration Update via 3GPP access and indicates that it wants to activate UP connections using "PDU Sessions to be activated" parameter, it applies in the same way for single-access and MA PDU Sessions. It is proposed here that the same is assumed for mobility Registration via non-3GPP access.

### 6.16.3	Procedures

This procedure assumes that the UE is registered over untrusted non-3GPP access first and then switches to trusted non-3GPP access. A similar procedure is applicable if UE is registered over trusted non-3GPP access first and then switches to untrusted non-3GPP access. The UE may also be registered in 3GPP access. The solution re-uses some aspects of Solution#5.2, e.g. a new registration type.

Figure 6.16.2-1. Mobility Registration procedure to support non-3GPP access switching

1.	The UE is registered over an untrusted non-3GPP access and may also be registered in 3GPP access. A MA PDU Session is established that has UP connections via untrusted non-3GPP access and possibly also via 3GPP access. The UE may also have single access (SA) PDU Sessions over 3GPP and/or untrusted non-3GPP access.

During registration in non-3GPP, the AMF indicates to UE whether it supports mobility Registration in non-3GPP access.

NOTE:	It is assumed that network deployment ensures a consistent experience i.e. with homogenous support for mobility Registration in non-3GPP access.

During the MA PDU Session Establishment, the UE indicates whether it supports non-3GPP access switching in the PDU Session Establishment Request message. The AMF also indicates whether it supports non-3GPP access switching to the SMF. Considering the received capabilities of the UE, AMF and SMF capability, the SMF indicates whether non-3GPP access switching is supported to the UE in the PDU Session Establishment Accept message.

Editor's note:	It is FFS whether the above negotiation at MA PDU Session establishment is required.

2.	The UE registers over trusted non-3GPP access with new Registration type = non-3GPP access switching. The UE includes in the List Of PDU Sessions To Be Activated the MA PDU Sessions that have user plane resources established over untrusted non-3GPP access. The UE may also include SA PDU Sessions active in untrusted non-3GPP access in the List Of PDU Sessions To Be Activated.

Editor's note:	It is FFS whether a new Registration type is used, or it is better to use Registration type for mobility registration and add a separate indication.

3.	The AMF may perform authentication based on existing procedures. The AMF may send an Initial Context Setup Request to TNGF and include AS security information. Alternatively, the AMF sends the Initial Context Setup Request to TNGF in step 6, after contacting the SMF(s).

4.	The AMF notifies to the SMF that the UE requested UP connection activation. For MA PDU Sessions, the AMF also indicates that the purpose is for "non-3GPP access switching". The AMF repeats this step for all PDU Sessions indicated in step 2.

Editor's note:	It is FFS whether SMF needs a special behaviour for "non-3GPP access switching" or whether the solution works also without this indication.

5.	The SMF contacts UPF to allocate CN tunnel information, if needed. The SMF replies to AMF including the CN tunnel information.

6.	The AMF sends an N2 message to TNGF and includes the CN tunnel information received from the SMF(s). The N2 message is an Initial Context Setup Request (if not sent in step 3) or a PDU Session Resource Setup request. The AMF knows that the N2 information is targeting TNGF since the UP connection setup is part of the registration in trusted non-3GPP access.

The TNGF completes the IPSec SA establishment as described in clause 4.12a.2.2 of TS 23.502 [3] and triggers establishment of UP resources towards the UE, e.g. to create a first Child SA for the PDU Session.

7.	The TNGF replies with Initial Context Setup Response, including AN tunnel information.

8.	The AMF forwards the information to the SMF(s) and the SMF(s) provides the tunnel information to the UPF(s). For MA PDU Sessions, the UPF starts to use the trusted non-3GPP access as the non-3GPP access for the MA PDU Session, i.e. untrusted non-3GPP access is not used for downlink traffic anymore. For SA PDU Sessions, the downlink tunnel is switched to the trusted non-3GPP access. The UPF still accepts uplink packets from the untrusted non-3GPP access.

9.	The AMF sends a NAS Registration Accept message to the UE, via the TNGF.

10.	The AMF updates the registration for non-3GPP access in UDM, by providing the new RAT type.

11.	The UE may send a NAS Registration Complete message, as described in TS 23.502 [3].

12.	The AMF triggers a AN release of the untrusted non-3GPP access. As a result of this procedure, user plane resources over untrusted non-3GPP access of MA PDU is completely released. Also user plane resources of SA PDU Session(s) are deactivated. The N3IWF does not need to signal to the UE since the UE may not be reachable over the old non-3GPP access. Latest when the UE recognize that user plane resources over untrusted non-3GPP access are released, the UE starts to send traffic over trusted non-3GPP access.

Editor's note: It is FFS whether the UE should switch to trusted non-3GPP access already after step 6, i.e. directly after the UP resources in trusted non-3GPP access are available.

### 6.16.4	Impacts on Existing Nodes and Functionality

UE:

-	Performs Registration with new registration type.

-	Indicates to the SMF that the UE supports non-3GPP access switching during the MA PDU Session Establishment.

-	Temporarily maintains simultaneous parallel user plane tunnel over untrusted 3GPP access and trusted non-3GPP access.

-	When the UE receives the Registration Accept message for the target non-3GPP access, the UE considers that it is deregistered from the source non-3GPP access.

AMF:

-	During the Registration procedure, indicate capability for mobility Registration via non-3GPP access to UE, and delay UDM update until non-3GPP access switching is completed.

-	Indicates to the SMF that the AMF supports non-3GPP access switching during the MA PDU Session Establishment.

-	Notifies to the SMF that the UE requested non-3GPP access switching during the Registration procedure.

SMF:

-	Indicates to the UE whether the MA PDU Session supports non-3GPP access switching during the MA PDU Session Establishment.

UPF:

-	Ability to handle a change of AN TEID for the non-3GPP leg

UDM:

-	Support for updating RAT type using Nudm_UECM Update.

## 6.17	Solution #5.6: Consolidated solution for traffic switching between two non-3GPP access paths

### 6.17.1	Introduction

The solution in clause 6.X addresses the objective of KI#5, i.e. it specifies how the data traffic of an MA PDU Session can be switched between two non-3GPP access paths, both of them using the same PLMN. The solution borrows several aspects from solutions #5.2, #5.3 and #5.4.

### 6.17.2	High-level Description

The key principles of the solutions are listed below and are further detailed in the next clause.

a.	The UE applies the existing procedures to register to 5GC via a non-3GPP access network. Optionally, the UE may also register to 5GC via a 3GPP access network (e.g. via NG-RAN).

b.	An MA PDU Session is established based on the existing procedures specified in TS 23.502 [3], clause 4.22.2. In addition, during the MA PDU Session establishment, the UE and the network indicate whether they support "Non-3GPP path switch", i.e. whether they can switch the traffic of an MA PDU Session from one Non-3GPP access path to another Non-3GPP access path.

c.	If both the UE and the network support "Non-3GPP path switch", then the following steps apply:

i)	When the UE wants to switch the traffic of the MA PDU Session from the existing non-3GPP access path to the new non-3GPP access path, the UE initiates registration over the new non-3GPP access path and indicates that the registration is for "Non-3GPP path switch". Also, the UE includes in the "List Of PDU Sessions To Be Activated" the identity of the existing MA PDU Session.

ii)	A registration procedure (based on TS 23.502 [3], clause 4.2.2.2) takes place over the new non-3GPP access path and the SMF triggers establishment of user-plane resources for the MA PDU Session over the new non-3GPP access path. At this point, the MA PDU Session has user-plane resources over the old non-3GPP access path and over the new non-3GPP access path.

iii)	The AMF updates its registration with the UDM for non-3GPP access and initiates deregistration over the old non-3GPP access path. The deregistration releases the user-plane resources over the old non-3GPP access and the UE and UPF start using the new non-3GPP access for the MA PDU Session.

In order to support "Non-3GPP path switch" in a PLMN, it is assumed that all AMFs supporting ATSSS in the PLMN are enhanced to support "Non-3GPP path switch". This ensures that, if the UE establishes an MA PDU Session when connected to one AMF and receives an indication that "Non-3GPP path switch" is supported, the UE can assume the "Non-3GPP path switch" is still supported after moving into the serving area of another AMF in the same PLMN.

### 6.17.3	Procedures

Figure 6.17.3-1 below depicts the key steps of the solution, which enables the data traffic of an MA PDU Session to be switched from a non-3GPP access path using a N3IWF to a non-3GPP access path using a TNGF. The same steps can be used to switch the data traffic of an MA PDU Session between any non-3GPP access paths (using either N3IWF or TNGF).

Figure 6.17.3-1: Procedure for enabling traffic switching between two non-3GPP access paths

1.	Optionally, the UE performs an initial 5G registration over 3GPP access in a PLMN. The selected AMF registers with UDM for 3GPP access and provides its GUAMI and RAT type = NR.

2.	The UE selects an N3IWF and performs an initial 5G registration over untrusted non-3GPP access in the same PLMN. The same AMF is selected, as the one in the previous step. The AMF registers with UDM for non-3GPP access and provides its GUAMI and RAT type = Untrusted WLAN.

3.	The UE requests an MA PDU Session, as specified in TS 23.502, clause 4.22.2. Since the UE supports Non-3GPP path switch, the UE includes in the UL NAS Transport message a Non-3GPP path switch supported indication. Based on this indication, the AMF selects an SMF that supports Non-3GPP path switch and sends a Create SM Context Request to SMF including a Non-3GPP path switch indication which indicates that the AMF and the UE support Non-3GPP path switch. The SMF registers with the UDM and additional steps are executed to complete the MA PDU Session establishment including the establishment of user-plane resource over 3GPP access and over untrusted non-3GPP access.

The PDU Session Establishment Accept message sent to UE indicates that Non-3GPP path switch is supported for the MA PDU Session.

4.	The UE detects a trusted non-3GPP access network that supports 5G connectivity to the same PLMN. The UE decides to switch the data traffic transferred over the untrusted non-3GPP access of the MA PDU Session to the detected trusted non-3GPP access network.

5.	For this purpose, the UE initiates a 5G registration over trusted non-3GPP access and sends a Registration Request to the same AMF. The Registration Request contains a Non-3GPP path switch indication (e.g. either a new registration type = "Non-3GPP path switch" or a registration type = "Initial" plus a new IE), which indicates that the registration over trusted non-3GPP access is required for switching the data traffic of an MA PDU Session to another non-3GPP access path. The Non-3GPP path switch indication indicates to AMF that the existing registration via untrusted non-3GPP access should not be released until the path switch is completed.

In the "List of PDU Sessions To Be Activated" the UE includes the identity of the established MA PDU Session in order to trigger the establishment of user-plane resources over the trusted non-3GPP access.

6.	Optionally, an authentication procedure may be executed.

7.	The normal SMC Request / Response messages are exchanged to setup NAS security between the UE and the AMF over the trusted non-3GPP access.

8.	The AMF sends an Initial Context Setup Request to TNGF including the TNGF key, which triggers the establishment of the NWt connection between the UE and the TNGF.

9.	Since the UE provided a "List of PDU Sessions To Be Activated" including the identity of the established MA PDU Session, the AMF sends an Update SM Context Request to SMF to initiate the establishment of user-plane resources over the trusted non-3GPP access for this MA PDU Session. The Update SM Context Request contains a Non-3GPP path switch indication which indicates that the requested update is for switching the data traffic of an MA PDU Session to the trusted non-3GPP access and that the SMF shall not release the existing user-plane resources over the untrusted non-3GPP access. These resources are released later, in step 13.

10.	The user-plane resources over the trusted non-3GPP access are established. At this point, the MA PDU Session has user-plane resources over the untrusted non-3GPP access path, over the trusted non-3GPP access path, and (optionally) over the 3GPP access path. The UE and the UPF may switch the traffic of the MA PDU Session from the untrusted non-3GPP access path to the trusted non-3GPP access path. Alternatively, this traffic switching can be performed in step 14.

Editor's note:	Whether the UE and UPF perform path switching in step 10 (right after the establishment of UP resources on the trusted non-3GPP access), on in step 14 (right after the release of UP resources on the untrusted non-3GPP access) needs further consideration.

11.	The AMF updates its UDM registration for non-3GPP access by sending a UECM Registration Request to UDM including RAT type = Trusted WLAN.

12.	Based on the current procedures, the UDM accepts the new registration for non-3GPP access and notifies the AMF that the previous registration for non-3GPP access (performed in step 2) is now deregistered.

13.	After updating the UDM, the AMF triggers an AN release procedure towards the N3IWF to release the N2 connection and the user-plane resources over the untrusted non-3GPP access.

14.	After the user-plane resources over the untrusted non-3GPP access are released, and if the UE and UFP have not switched the traffic to the trusted non-3GPP access in step 10, then the UE and the UPF use the trusted non-3GPP access to transfer the MA PDU Session traffic that should be sent over non-3GPP access. This completes the non-3GPP path switching from the untrusted non-3GPP access to the trusted non-3GPP access.

15.	Finally, the registration procedure is completed by sending a Registration Accept message to the UE. When the UE receives the Registration Accept message, it considers that it is deregistered from the untrusted non-3GPP access, and it is registered over the trusted non-3GPP access.

### 6.17.4	Impacts on Existing Nodes and Functionality

UE:

-	During the MA PDU Session Establishment, the UE indicates that it supports non-3GPP path switch.

-	Performs registration by providing a Non-3GPP path switch indication. This indication can be provided either as a new registration type or as a new IE.

-	Temporarily maintains simultaneous user-plane resources over the untrusted 3GPP access and over the trusted non-3GPP access.

-	When the UE receives the Registration Accept message for the new non-3GPP access, the UE considers that it is deregistered from the old non-3GPP access.

AMF:

-	During the Registration procedure, the AMF updates the UDM registration for non-3GPP access after the user-plane resources over the new non-3GPP access are established.

-	Indicates to the SMF that the AMF and the UE support non-3GPP path switch during the MA PDU Session Establishment.

-	Indicates to SMF that the UE requested non-3GPP path switch during the Registration procedure.

-	Performs AN release over old access during the Registration procedure

SMF:

-	Indicates to UE whether the MA PDU Session supports non-3GPP path switch during the MA PDU Session Establishment.

UPF:

-	Temporarily maintains simultaneous user-plane tunnels over untrusted non-3GPP access and over trusted non-3GPP access.

## 6.X	Solution #X: <Solution Title>

### 6.X.1	Introduction

Editor's note:	This clause makes a brief introduction of the solution, and provides the key aspects of the solution as well as the principles and the architecture assumptions of the solution.

### 6.X.2	High-level Description

Editor's note:	This clause describes the solution in more detail. (Sub) clause(s) may be added to describe different aspects of the solution.

### 6.X.2	Procedures

Editor's note:	This clause describes the procedures and information flows for the solution.

### 6.X.3	Impacts on Existing Nodes and Functionality

Editor's note:	This clause captures impacts on existing 3GPP nodes and functional elements.

# 7	Evaluation

Editor's note:	This clause provides the evaluation of different solutions. It may contain an evaluation for each Key Issue.

## 7.1	Evaluation for KI #6: Support non-3GPP access leg of MA-PDU Session with PDN connection in EPC

Solution #6.1 addresses KI#6 to support the non-3GPP access leg of MA-PDU Session with PDN connection in EPC. This solution also supports the mobility procedure from 5GS to EPS. It applies the similar mechanism as the solution for MA-PDU session with one leg via non-3GPP access to 5GC and one leg via 3GPP access to EPC, as defined in Rel-17, with a bit impact on the ePDG and PGW-C+SMF to support APCO IE to transport the ATSSS related parameters.

# 8	Conclusions

Editor's note:	This clause enlists the conclusions of the study, including agreements for work to be done in the normative phase.

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-02 | SA2#149E | S2-2200755 | - | - | - | Skeleton | 0.0.0 |
| 2022-02 | SA2#149E | S2-2200757 | - | - | - | Scope of TR 23.700-53 | 0.1.0 |
| 2022-02 | SA2#149E | S2-2201334 | - | - | - | Architectural assumptions for ATSSS_Ph3 | 0.1.0 |
| 2022-02 | SA2#149E | S2-2201335 | - | - | - | KI for WT#5.1 | 0.1.0 |
| 2022-02 | SA2#149E | S2-2201336 | - | - | - | Key Issue for supporting redundant traffic steering | 0.1.0 |
| 2022-02 | SA2#149E | S2-2201337 | - | - | - | KI on 'New steering functionalities for non-TCP traffic' | 0.1.0 |
| 2022-02 | SA2#149E | S2-2201338 | - | - | - | KI for WT#6 | 0.1.0 |
| 2022-02 | SA2#149E | S2-2201339 | - | - | - | KI and solution for support of redundant traffic steering | 0.1.0 |
| 2022-02 | SA2#149E | S2-2201340 | - | - | - | Solution for Multi-access between EPC and 5GC | 0.1.0 |
| 2022-02 | SA2#149E | S2-2201341 | - | - | - | New Solution: MP-DCCP-LL based ATSSS steering functionality for Non-TCP traffic | 0.1.0 |
| 2022-04 | SA2#150E | S2-2203066 | - | - | - | Update of Solution 3 MP-DCCP-LL based ATSSS steering functionality | 0.2.0 |
| 2022-04 | SA2#150E | S2-2203067 | - | - | - | Solution #X: MPQUIC steering functionality using UDP proxying over HTTP | 0.2.0 |
| 2022-04 | SA2#150E | S2-2203068 | - | - | - | Solution #X: MPQUIC steering functionality using IP proxying over HTTP | 0.2.0 |
| 2022-04 | SA2#150E | S2-2203069 | - | - | - | New Solution: Limiting MA-PDU Per-Packet Overhead | 0.2.0 |
| 2022-04 | SA2#150E | S2-2203070 | - | - | - | New solution on redundant traffic steering triggered by AF | 0.2.0 |
| 2022-04 | SA2#150E | S2-2203071 | - | - | - | Update solution for redundancy steering mode | 0.2.0 |
| 2022-04 | SA2#150E | S2-2203072 | - | - | - | Solution for support of redundant traffic steering | 0.2.0 |
| 2022-04 | SA2#150E | S2-2203073 | - | - | - | Solutions for KI#3 Redundant Traffic Steering Mode | 0.2.0 |
| 2022-04 | SA2#150E | S2-2203074 | - | - | - | KI#3: Solution for redundant steering mode with duplication information and trigger mechanisms | 0.2.0 |
| 2022-04 | SA2#150E | S2-2203075 | - | - | - | Solution for supporting traffic switching between two N3GPP paths | 0.2.0 |
| 2022-04 | SA2#150E | S2-2203076 | - | - | - | KI #5, New Sol: Delaying UDM Registration until non-3GPP access switching completes | 0.2.0 |
| 2022-04 | SA2#150E | S2-2203077 | - | - | - | Solution #X: KI#5 - Path switching between Non-3GPP accesses | 0.2.0 |
| 2022-04 | SA2#150E | S2-2203078 | - | - | - | Solution for KI#5 Switching traffic of an MA PDU Session between two non-3GPP access paths | 0.2.0 |
| 2022-04 | SA2#150E | S2-2203079 | - | - | - | Update solution for Multi-access between EPC and 5GC | 0.2.0 |
| 2022-05 | SA2#151E | S2-2204753 | - | - | - | Update of KI#5 | 0.3.0 |
| 2022-05 | SA2#151E | S2-2204483 | - | - | - | Inner Header Compression for Solution #2.1 | 0.3.0 |
| 2022-05 | SA2#151E | S2-2204754 | - | - | - | Update of solution #2.2: MPQUIC steering functionality using UDP proxying over HTTP | 0.3.0 |
| 2022-05 | SA2#151E | S2-2204755 | - | - | - | Update of solution #2.3: MPQUIC steering functionality using IP proxying over HTTP | 0.3.0 |
| 2022-05 | SA2#151E | S2-2204756 | - | - | - | Update of solution #2.4: Limiting MA-PDU Per-Packet Overhead | 0.3.0 |
| 2022-05 | SA2#151E | S2-2203739 | - | - | - | New solution for KI#3 on redundant traffic steering | 0.3.0 |
| 2022-05 | SA2#151E | S2-2204905 | - | - | - | KI#3, Update solution #3.2 for redundant traffic steering triggered by AF | 0.3.0 |
| 2022-05 | SA2#151E | S2-2204906 | - | - | - | KI#3, Sol #3.5: Update to address ENs | 0.3.0 |
| 2022-05 | SA2#151E | S2-2204638 | - | - | - | KI#3, Sol #3.3 update: New traffic duplication steering mode | 0.3.0 |
| 2022-05 | SA2#151E | S2-2204907 | - | - | - | KI#3: Update of solution 3.4 | 0.3.0 |
| 2022-05 | SA2#151E | S2-2204757 | - | - | - | KI#5, New Sol: Solution for switching traffic between non-3GPP accesses | 0.3.0 |
| 2022-05 | SA2#151E | S2-2204758 | - | - | - | KI#5, Update solution #5.1 to address the ENs | 0.3.0 |
| 2022-05 | SA2#151E | S2-2204759 | - | - | - | Update of Solution #5.3: KI#5 - Path switching between Non-3GPP accesses addressing ENs | 0.3.0 |
| 2022-05 | SA2#151E | S2-2203957 | - | - | - | Update of Solution #5.3: KI#5 - Path switching between Non-3GPP accesses addressing EN | 0.3.0 |
| 2022-05 | SA2#151E | S2-2204639 | - | - | - | KI#5, Sol #5.3 update: Path switching between non-3GPP accesses | 0.3.0 |
| 2022-05 | SA2#151E | S2-2204224 | - | - | - | KI#5, Sol#5.4: Updates to resolve editor s notes | 0.3.0 |
| 2022-05 | SA2#151E | S2-2204760 | - | - | - | KI#5, Sol #5.2 Update to resolve EN and to support AMF change scenario | 0.3.0 |
| 2022-05 | SA2#151E | S2-2204761 | - | - | - | Consolidated solution for KI#5 | 0.3.0 |
| 2022-05 | SA2#151E | S2-2204253 | - | - | - | KI#6, Sol #6.1: Update solution and add evaluation for KI#6 | 0.3.0 |