| 3GPP TR 23.700-57 V0.1.0 (2025-04) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on application enablement aspects for MMTel Phase 2 (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	7

3.3	Abbreviations	7

4	Key Issues	7

4.1	Key Issue #1: Alignment eMMTel service flows with SA2 Rel-19 support for IMS Capability Exposure Framework	7

4.1.1	Description	7

4.2	Key Issue #2: DC Application list search	8

4.2.1	Description	8

4.2.2	Open Issue	8

4.3	Key Issue #3: enablement of standalone DC	9

4.3.1	Description	9

4.3.2	Open Issue	9

4.4	Key Issue #4: Northbound interface of DCAR	9

4.4.1	Description	9

4.4.2	Open Issue	9

4.x	Key Issue #X: <Key Issue Title>	9

4.x.1	Description	9

4.x.2	Open Issue	9

5	Architectural requirements	10

6	Solutions	10

6.X	Solution #X: <Solution Title>	10

6.X.1	Description	10

6.X.2	Procedures	10

6.X.3	Impacts on Existing Nodes and Functionality	10

7	Overall Evaluation	10

7.1	General	10

7.2	Mapping of Solutions to Key Issues	10

7.3	Solution evaluation	11

8	Conclusions	11

Annex A: Change history	11

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

# x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

# 1	Scope

The present document studies the following aspects:

-	the solutions to solve the remaining issues in Rel-19 MMTel_APP WI, including:

i.	support the enablement of MMTel calling with or without Data Channel using existing APIs provided by OMA and capture the gaps; and

ii.	providing capabilities to Application to enable MMTel related value added service; and

-	the application enablement aspects for MMTel service based on the specifications developed by SA2 in Rel-19 (NEF, NG_RTC_Ph2 WI).

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.392: "Application enablement aspects for MMTel".

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[4]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS)".

[5]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

.

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Key Issues

## 4.1	Key Issue #1: Alignment eMMTel service flows with SA2 Rel-19 support for IMS Capability Exposure Framework

### 4.1.1	Description

During the normative work performed in 3GPP TS 23.392 [2], there were notes related to further alignment needed with SA2 work, for example:

-	Clause 6.2.2 “MMTel Enabler Server architecture model supporting IMS Session Management” :

“NOTE 1:	The MMTel Enabler Server architecture model supporting IMS Session Management will be updated after the IMS Session management capabilities provided by NEF is specified by SA2.”

-	Clause 8.4.2.2.1 “Establishing an Application Call with Data Channel capability between non-IMS application and DCMTSI client”:

“Editor's note:	The flow will need to be updated once SA2 work is done to specify NEF exposure of IMS session management (Nnef_ImsSessionManagement).”

-	Clause 8.4.3.2.1	 “Establishing a Third-Party Call with Data Channel capability”:

“Editor's note:	The flow will need to be updated once SA2 work is done to specify NEF exposure of IMS session management (Nnef_ImsSessionManagement).”

Based on the Rel-19 specifications, the following IMS related exposure capabilities have been specified and need to be analysed on how they impact the eMMTel service flows.

In 3GPP TS 23.228 [4], the normative Annex AG has specified the “IMS Capability Exposure Framework in IMS” which provided exposure to non-trusted applications with following capabilities:

-	Managing data channel for an on-going IMS session (i.e., adding, removing or updating the data channel)

-	Creating a standalone data channel session, and

-	Terminating a standalone data channel session.

The procedures can be triggered via the NEF on the originating or terminating side.

In 3GPP TS 23.502 [3], the following capabilities are exposed by the NEF via N33 reference point:

-	IMS Event Exposure service (Nnef_ImsEventExposure service) with support for subscribe, unsubscribe and notify,

-	IMS Session management service (Nnef_ImsSessionManagement service) with support for:

i.	creating an IMS session with requested media,

ii.	IMS session management update to allow changes to the media handled in the IMS session,

iii.	IMS session managememt delete which allows termination/release of an IMS session, and

iv.	the IMS session management notification which provides details of a subscribed event that occurred during an IMS session.

-	IMS parameter provisioning service (ImsPP) which exposes to an AF IMS parameter management capabilities (e.g. Rich Call Data (RCD) server address, RCD URL or RCD information), in order to:

i.	modify the IMS user related information for a target IMPU.

ii.	create the IMS user related information for the target IMPU, and

iii.	delete the provisioned IMS user related information for the target IMPU.

To support exposure of MMTel based services, including data channel media, the following aspects need to be studied:

1.	leveraging the Rel-19 support in the NEF as defined in 3GPP TS 23.502 [3] in clauses 5.2.6.39, 5.2.6.40 and 5.2.6.41 to identify the service flows where MMTel enabler server consumes exposed capabilities via NEF, and analyse if any updates needed for the MMTel enabler architecture.

2.	The exposure via the NEF is applicable to the MMTel enabler server in both trusted and untrusted AF cases.

3.	The direct exposure via the DCSF is applicable to the MMTel enabler server in the trusted AF cases only.

## 4.2	Key Issue #2: DC Application list search

### 4.2.1	Description

The DC application profiles related procedures, including downloading on UE and updating on UE are specified in 3GPP TS 23.392 [2]. In the downloading procedure, the MMTel Enabler Client requests for the specified number of application profiles (Number of application profiles required) of DC Applications, beginning from the Begin index included in the request. However, when an excessive number of DC Applications (threshold example: >20 instances) is available for a single call session, the conventional page-by-page navigation through the DC application list becomes impractical for end-users to locate their desired DC Application efficiently. Since the selection of DC application is heavily depends on the service policy of MMTel service provider, it is not suitable to leave this issue to implement specific. Therefore, providing a standardized mechanism (i.e. by enhancing the capability in MMTel-1 reference point) to search the DC Applications for user is necessary.

In addition, The DC Application may support conditional search as a value-added service provided by the MMTel Enabler, e.g. the DC Applications can be provided to the user based on the user’s location. This conditional search may also be provided to user by combing the existing SEAL services such as Location Management service. It is valuable to study how to provide conditional search for the DC Applications as a value added service to MMTel users.

### 4.2.2	Open Issue

This key issue aims to solve the following open issues in the MMTel Enabler layer:

1.	How to provide capability to MMTel Enabler Client to search the DC Applications.

2.	How to provide conditional search for the DC Applications as a value added service to MMTel users.

Editor’s note:	Whether the URL for downloading the DC Application list, as specified in 3GPP TS 26.114 [5], can be modified (e.g., by adding some parameters) is FFS, and coordination with SA4 is required.

## 4.3	Key Issue #3: enablement of standalone DC

### 4.3.1	Description

Standalone IMS Data Channel Sessions is specified in Annex AC.10 of 3GPP TS 23.228 [4], and the related IMS AS Services is specified in Annex AA.2.4 of 3GPP TS 23.228 [4].

For the A2P use cases, the Standalone Data Channel provides application providers and vertical service providers with a method to establish Data Channels and interact with users through them, without requiring the setup of audio/video calls. For these providers, in most usage scenarios, the primary focus is on delivering application content (via the Data Channel) rather than managing underlying IMS session details. Therefore, it is valuable for the MMTel Enabler to enable applications to directly use the application interaction capabilities of Standalone DC, such as sending and receiving of a content like menu, without needing to know whether an IMS Session exists in the network.

### 4.3.2	Open Issue

This key issue aims to solve the following open issue in the MMTel Enabler layer:

1.	How to provide applications with direct access to Standalone Data Channel capabilities for application-layer interactions (e.g., menu content exchange), abstracting away the underlying IMS session awareness requirements.

## 4.4	Key Issue #4: Northbound interface of DCAR

### 4.4.1	Description

The DC application and profile management on DCAR via MMTel Enabler Server is specified in 3GPP TS 23.392 [2] in Rel-19, but the northbound interface of DCAR used by MMTel Enabler Server to manage the DC application and profile is not specified. There are three remaining Editor's notes regarding this issue in clause 8.3.2.2, 8.3.3.2 and 8.3.4.2 in 3GPP TS 23.392 [2] respectively. The Editor’s note is:

Editor's note:	 The detailed interactions between the MMTel Enabler Server and DCAR are FFS.

Therefore, the northbound interface of DCAR is proposed to be studied in Rel-20.

### 4.4.2	Open Issue

This key issue aims to solve the following open issues in the MMTel Enabler layer:

1.	Study the northbound interface of DCAR, which is used to manage the DC applications in the DCAR by application providers.

2.	Whether the MMTel-3 reference point specified in 3GPP TS 23.392 [2] can be reused as the northbound interface of DCAR, or whether enhancement or modification is needed.

## 4.x	Key Issue #X: <Key Issue Title>

### 4.x.1	Description

This clause provides a description of the key issue.

### 4.x.2	Open Issue

This clause provides the open issue(s) of the key issue.

# 5	Architectural requirements

This clause provides the all architectural requirements identified.

# 6	Solutions

## 6.X	Solution #X: <Solution Title>

### 6.X.1	Description

This clause will describe the solution principles and architecture assumptions for corresponding key issue(s). (Sub) clause(s) may be added to capture details.

### 6.X.2	Procedures

This clause describes high-level procedures and information flows for the solution.

### 6.X.3	Impacts on Existing Nodes and Functionality

This clause captures impacts on existing 3GPP nodes and functional elements.

# 7	Overall Evaluation

This clause will provide evaluation of different solutions. The mapping between KIs and solutions will be also captured in this clause.

## 7.1	General

## 7.2	Mapping of Solutions to Key Issues

## 7.3	Solution evaluation

# 8	Conclusions

This clause will list conclusions that have been agreed during the  study item activities.

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-03 | SA6#66 |  | - | - | - | Proposed skeleton approved at S6#66 | 0.0.0 |
| 2025-04 | SA6#66 |  |  |  |  | Implemented pCRs approved in S6#66: S6-251200, S6-251203, S6-251362, S6-251363, S6-251364, S6-251217 Editorial changes by the rapporteur | 0.1.0 |