| 3GPP TR 23.700-57 V0.4.0 (2025-10) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on application enablement aspects for MMTel Phase 2 (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	8

3.3	Abbreviations	8

4	Key Issues	8

4.1	Key Issue #1: Alignment eMMTel service flows with SA2 Rel-19 support for IMS Capability Exposure Framework	8

4.1.1	Description	8

4.1.2	Open Issue	9

4.2	Key Issue #2: DC Application list search	9

4.2.1	Description	9

4.2.2	Open Issue	9

4.3	Key Issue #3: enablement of standalone DC	10

4.3.1	Description	10

4.3.2	Open Issue	10

4.4	Key Issue #4: Northbound interface of DCAR	10

4.4.1	Description	10

4.4.2	Open Issue	10

4.5	Key Issue #5: Support of virtual number	11

4.5.1	Description	11

4.5.2	Open Issue	13

4.6	Key Issue #6: application enablement aspects for A2P and P2A avatar communication	13

4.6.1	Description	13

4.6.2	Open Issue	14

4.x	Key Issue #X: <Key Issue Title>	14

4.x.1	Description	14

4.x.2	Open Issue	14

5	Architectural requirements	14

5.1	General	14

5. 2	Requirements	14

6	Solutions	15

6.1	Solution #1: DC Application list search	15

6.1.1	Description	15

6.1.2	Procedures	15

6.1.3	Impacts on Existing Nodes and Functionality	16

6.2	Solution #2: Northbound interface of DCAR	16

6.2.1	Description	16

6.2.2	Procedures	17

6.2.2.1	Uploading DC application and profile to DCAR	17

6.2.2.2	Updating DC application and profile to DCAR	18

6.2.2.3	Deleting DC application and profile from DCAR	19

6.2.3	Impacts on Existing Nodes and Functionality	21

6.3	Solution #3: Alignment 8.4.2 and 8.4.3 service flows with SA2 Rel-19 support for IMS Capability Exposure Framework	21

6.3.1	Description	21

6.3.2	Procedures	21

6.3.2.1	Update to existing procedures in 8.4.2	21

6.3.2.2	Update to existing procedures in 8.4.3	23

6.3.3	Impacts on Existing Nodes and Functionality	28

6.4	Solution #4: Enable the A2P avatar communication	28

6.4.1	Description	28

6.4.2	Procedures	28

6.4.3	Impacts on Existing Nodes and Functionality	29

6.5	Solution #5: Enablement of standalone DC	29

6.5.1	Description	29

6.5.2	Procedures	29

6.1.3	Impacts on Existing Nodes and Functionality	31

6.X	Solution #X: <Solution Title>	31

6.X.1	Description	31

6.X.2	Procedures	31

6.X.3	Impacts on Existing Nodes and Functionality	32

7	Overall Evaluation	32

7.1	General	32

7.2	Mapping of Solutions to Key Issues	32

7.3	Solution evaluation	32

8	Conclusions	32

Annex A: Change history	32

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

# x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

# 1	Scope

The present document studies the following aspects:

-	the solutions to solve the remaining issues in Rel-19 MMTel_APP WI, including:

i.	support the enablement of MMTel calling with or without Data Channel using existing APIs provided by OMA and capture the gaps; and

ii.	providing capabilities to Application to enable MMTel related value added service; and

-	the application enablement aspects for MMTel service based on the specifications developed by SA2 in Rel-19 (NEF, NG_RTC_Ph2 WI).

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.392: "Application enablement aspects for MMTel".

[3]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[4]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS)".

[5]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[6]	GSMA NG.110: "Multi Device".

[7]	3GPP TS 22.173: "IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services".

[8]	3GPP TS 23.438: "Service Enabler Architecture Layer for Verticals (SEAL); Digital assets".

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Key Issues

## 4.1	Key Issue #1: Alignment eMMTel service flows with SA2 Rel-19 support for IMS Capability Exposure Framework

### 4.1.1	Description

During the normative work performed in 3GPP TS 23.392 [2], there were notes related to further alignment needed with SA2 work, for example:

-	Clause 6.2.2 “MMTel Enabler Server architecture model supporting IMS Session Management” :

“NOTE 1:	The MMTel Enabler Server architecture model supporting IMS Session Management will be updated after the IMS Session management capabilities provided by NEF is specified by SA2.”

-	Clause 8.4.2.2.1 “Establishing an Application Call with Data Channel capability between non-IMS application and DCMTSI client”:

“Editor's note:	The flow will need to be updated once SA2 work is done to specify NEF exposure of IMS session management (Nnef_ImsSessionManagement).”

-	Clause 8.4.3.2.1	 “Establishing a Third-Party Call with Data Channel capability”:

“Editor's note:	The flow will need to be updated once SA2 work is done to specify NEF exposure of IMS session management (Nnef_ImsSessionManagement).”

Based on the Rel-19 specifications, the following IMS related exposure capabilities have been specified and need to be analysed on how they impact the eMMTel service flows.

In 3GPP TS 23.228 [4], the normative Annex AG has specified the “IMS Capability Exposure Framework in IMS” which provided exposure to non-trusted applications with following capabilities:

-	Managing data channel for an on-going IMS session (i.e., adding, removing or updating the data channel)

-	Creating a standalone data channel session, and

-	Terminating a standalone data channel session.

The procedures can be triggered via the NEF on the originating or terminating side.

In 3GPP TS 23.502 [3], the following capabilities are exposed by the NEF via N33 reference point:

-	IMS Event Exposure service (Nnef_ImsEventExposure service) with support for subscribe, unsubscribe and notify,

-	IMS Session management service (Nnef_ImsSessionManagement service) with support for:

i.	creating an IMS session with requested media,

ii.	IMS session management update to allow changes to the media handled in the IMS session,

iii.	IMS session management delete which allows termination/release of an IMS session, and

iv.	the IMS session management notification which provides details of a subscribed event that occurred during an IMS session.

-	IMS parameter provisioning service (ImsPP) which exposes to an AF IMS parameter management capabilities (e.g. Rich Call Data (RCD) server address, RCD URL or RCD information), in order to:

i.	modify the IMS user related information for a target IMPU.

ii.	create the IMS user related information for the target IMPU, and

iii.	delete the provisioned IMS user related information for the target IMPU.

To support exposure of MMTel based services, including data channel media, the following aspects need to be studied:

1.	leveraging the Rel-19 support in the NEF as defined in 3GPP TS 23.502 [3] in clauses 5.2.6.39, 5.2.6.40 and 5.2.6.41 to identify the service flows where MMTel enabler server consumes exposed capabilities via NEF, and analyse if any updates needed for the MMTel enabler architecture.

2.	The exposure via the NEF is applicable to the MMTel enabler server in both trusted and untrusted AF cases.

3.	The direct exposure via the DCSF is applicable to the MMTel enabler server in the trusted AF cases only.

### 4.1.2	Open Issue

This key issue aims to solve the following open issues in the MMTel Enabler layer:

1.	study the update of the flows and realization of the current features of the MMTel Enabler layer, reusing the IMS related capabilities exposed via NEF in 3GPP TS 23.502 [3].

2.	clarify which IMS capabilities are consumed in case of untrusted AF (via NEF) versus trusted AF (via NEF or directly from DCSF).

## 4.2	Key Issue #2: DC Application list search

### 4.2.1	Description

The DC application profiles related procedures, including downloading on UE and updating on UE are specified in 3GPP TS 23.392 [2]. In the downloading procedure, the MMTel Enabler Client requests for the specified number of application profiles (Number of application profiles required) of DC Applications, beginning from the Begin index included in the request. However, when an excessive number of DC Applications (threshold example: >20 instances) is available for a single call session, the conventional page-by-page navigation through the DC application list becomes impractical for end-users to locate their desired DC Application efficiently. Since the selection of DC application is heavily depends on the service policy of MMTel service provider, it is not suitable to leave this issue to implement specific. Therefore, providing a standardized mechanism (i.e. by enhancing the capability in MMTel-1 reference point) to search the DC Applications for user is necessary.

In addition, The DC Application may support conditional search as a value-added service provided by the MMTel Enabler, e.g. the DC Applications can be provided to the user based on the user’s location. This conditional search may also be provided to user by combing the existing SEAL services such as Location Management service. It is valuable to study how to provide conditional search for the DC Applications as a value added service to MMTel users.

### 4.2.2	Open Issue

This key issue aims to solve the following open issues in the MMTel Enabler layer:

1.	How to provide capability to MMTel Enabler Client to search the DC Applications.

2.	How to provide conditional search for the DC Applications as a value added service to MMTel users.

Editor’s note:	Whether the URL for downloading the DC Application list, as specified in 3GPP TS 26.114 [5], can be modified (e.g., by adding some parameters) is FFS, and coordination with SA4 is required.

## 4.3	Key Issue #3: enablement of standalone DC

### 4.3.1	Description

Standalone IMS Data Channel Sessions is specified in Annex AC.10 of 3GPP TS 23.228 [4], and the related IMS AS Services is specified in Annex AA.2.4 of 3GPP TS 23.228 [4].

For the A2P use cases, the Standalone Data Channel provides application providers and vertical service providers with a method to establish Data Channels and interact with users through them, without requiring the setup of audio/video calls. For these providers, in most usage scenarios, the primary focus is on delivering application content (via the Data Channel) rather than managing underlying IMS session details. Therefore, it is valuable for the MMTel Enabler to enable applications to directly use the application interaction capabilities of Standalone DC, such as sending and receiving of a content like menu, without needing to know whether an IMS Session exists in the network.

### 4.3.2	Open Issue

This key issue aims to solve the following open issue in the MMTel Enabler layer:

1.	How to provide applications with direct access to Standalone Data Channel capabilities for application-layer interactions (e.g., menu content exchange), abstracting away the underlying IMS session awareness requirements.

## 4.4	Key Issue #4: Northbound interface of DCAR

### 4.4.1	Description

The DC application and profile management on DCAR via MMTel Enabler Server is specified in 3GPP TS 23.392 [2] in Rel-19, but the northbound interface of DCAR used by MMTel Enabler Server to manage the DC application and profile is not specified. There are three remaining Editor's notes regarding this issue in clause 8.2.2.2, 8.2.3.2 and 8.2.4.2 in 3GPP TS 23.392 [2] respectively. The Editor’s note is:

Editor's note:	 The detailed interactions between the MMTel Enabler Server and DCAR are FFS.

There are other services of  DCAR that can be applied by Application servers, such as to provide DC application usage report from DCAR to Application servers through MMTel Enabler Server (e.g. downloading count of specific DC application during certain time period, etc.).

Therefore, the northbound interface of DCAR is proposed to be studied in Rel-20.

### 4.4.2	Open Issue

This key issue aims to solve the following open issues in the MMTel Enabler layer:

1.	Study the northbound interface of DCAR, which is used to manage the DC applications in the DCAR by application providers.

2.	Whether the MMTel-3 reference point specified in 3GPP TS 23.392 [2] can be reused as the northbound interface of DCAR, or whether enhancement or modification is needed.

## 4.5	Key Issue #5: Support of virtual number

### 4.5.1	Description

A virtual number is a widely used value-added calling service where the user's real number is hidden by the service provider to protect their privacy, especially in the A2P/P2A scenarios. This virtual number is used by one or both parties in a call and is not used in multi-device scenario, i.e. virtual number is not used by the secondary devices, as specified in GSMA PRD NG.110 [6]. This feature is also differs from the third party user identity in IMS sessions specified in Annex AF of 3GPP TS 23.228 [4] which allows the called party to use a third-party-specific identity in an IMS session.

This feature differs from the existing Terminating Identification Restriction (TIR) service specified in 3GPP TS 22.173 [7]. The virtual number looks like a real phone number rather than an anonymous number. A Virtual number may either pre-configured to the caller or assigned to the caller dynamically on demand of a user. There may be one or multiple virtual numbers for the same caller/callee in different calls or some caller may share a same virtual number. In addition, in some cases the callee can use the virtual number to call the caller back after their call is finished or to call the caller back when the call is not answered, but in some cases the virtual number cannot used to call back the caller.

Some examples of virtual number usage in different use cases are listed below. (This is not an exhaustive list and the arrows in the figures below points from the caller to the callee.):

Figure 4.5.1-1: AX mode of virtual number

a)	AX mode of virtual number is shown in Figure 4.5.1-1. When a caller A1 wants to hide its real number in every call, A1 can request a virtual number from the MMTel service provider, i.e. MNO, in AX mode. The MMTel service provider will assign a virtual number, e.g. a MSISDN for this specific usage, to A1. This virtual number applies to A1 in every call until A1 cancels this service from MMTel service provider. If AX mode virtual number applies, the following characteristics apply to the call:

-	outgoing calls: when A1 calls other users, other users, e.g. B1, B2 and B3, will see the caller number is x1 and number of A1 will be hid;

-	incoming calls: when other users call the virtual number x1, the call will go to A1.

Figure 4.5.1-2: AXB mode of virtual number

b)	AXB mode of virtual number is shown in Figure 4.5.1-2. In this mode, the virtual number applies to a specific caller-callee pair. If user A1 wants to hide its real number in the call with user B1only, A1 can request a virtual number x1 from the MMTel service provider, i.e. MNO, in AXB mode. If user A2 wants to hide its real number in the call with user B2 only, A2 can also requests a virtual number x1 from the MMTel service provider, i.e. MNO, in AXB mode. The virtual number x1 can be re-used in different caller-callee pair and applies to all calls between this caller-callee pair until A1 or A2 cancels this service from MMTel service provider. If AXB mode virtual number applies, the following characteristics apply to the call:

-	outgoing calls: when A1 calls B1, the callee B1 will see the caller number is x1 and number of A1 will be hid, when A1 calls other callees, the callees will see the real number of A1. Similarly, when A2 calls B21, the callee B2 will see the caller number is x1 and number of A2 will be hid, when A2 calls other callees, the callees will see the real number of A2.

-	incoming calls: when B1 calls the virtual number x1, the call will go to A1. Similarly, when B2 calls the virtual number x1, the call will go to A2. If users other than B1 and B2 call the virtual number x1, the call will be rejected by the MMTel service provider, i.e. MNO.

-	When A1 wants to use virtual number in the call with different users, e.g. B1, B2 or B3, different virtual number x1, x2 and x3 are needed to be requested from the MMTel service provider.

Figure 4.5.1-3: XB mode of virtual number

c)	XB mode of virtual number is shown in Figure 4.5.1-3. This mode may used by a group of users in a enterprise. If a enterprise wants to hide the every single number in the enterprise and different members in the enterprise can share the same virtual number in different time, the enterprise can request a virtual number from the MMTel service provider, i.e. MNO, in XB mode. This virtual number applies to the group of users in the enterprise until the enterprise cancels this service from MMTel service provider. If XB mode virtual number applies, the following characteristics apply to the call:

-	outgoing calls: XB mode can not apply.

-	incoming calls: when other users call the virtual number x1, the call will go to A1, A2 or A3 respectively in different time.

Figure 4.5.1-4: AXE mode of virtual number

d)	AXE mode of virtual number is shown in Figure 4.5.1-4. This mode may used by a group of users in a enterprise to reduce the cost of number and the call can reach every specific member in the enterprise. The enterprise can request  a virtual number and a series of extension from the MMTel service provider, i.e. MNO, in AXE mode. This virtual number plus the extensions applies to the group of users in the enterprise until the enterprise cancels this service from MMTel service provider. If AXE mode virtual number applies, the following characteristics apply to the call:

-	outgoing calls: AXE mode can not apply.

-	incoming calls: when other users call the virtual number x1 plus extension 101, 102 or 103, the call will go to A1, A2 or A3 respectively.

Figure 4.5.1-5: AXYB mode of virtual number

e)	AXYB mode of virtual number is shown in Figure 4.5.1-5. This mode is an enhancement of AX mode. When both caller A1 and callee B1 want to hide the real number in every call, A1 and B1 can request a virtual number x1 and y1 from the MMTel service provider respectively, i.e. MNO, in AXYB mode. In this mode, A1 and B1 do not know the real number of the other party. If AXYB mode virtual number applies, the following characteristics apply to the call:

-	outgoing calls: when A1 wants to call B1 or B2, A1 should call the number y1 or y2 respectively. B1 and B2 will see the caller number is x1 and number of A1 will be hid;

-	incoming calls: when B1 or B2 wants to call A1, B1 or B2 should call the number x1.A1 will see virtual number y1 or y2 respectively and number of B1 and B2 will be hid.

The user may request the network to assign virtual number(s), and the enterprise may request the network to assign a virtual number(s) for its employee(s). The virtual number(s) will be assigned in one or multiple modes. The network may assign a virtual number to the user on demand, and maintains the mapping between the user’s real number and the virtual number according to the requested mode(s). Considering the high flexibility requirement of the virtual number assignment and maintaining, this feature is not suitable to be handled in IMS.

### 4.5.2	Open Issue

This key issue aims to study how to support the virtual number service in the MMTel Enabler layer:

1.	How to manage the virtual number in MMTel Enabler layer, e.g. how to assign a virtual number(s) on demand of a user in one or multiple requested mode.

2.	How to maintain the relationship between the virtual number and real number in the requested mode(s) and the related number translation procedures in the call.

Editor's note 1:	Whether the capabilities specified in Annex AA.2.4 of 3GPP 23.228 can support the virtual number studied by SA6 is FFS. Coordination with SA2 is needed.

Editor's note 2	Whether the virtual number related procedures have impact on the procedures in 5GC/IMS is FFS and coordination with SA2 is needed.

Editor's note 3	When the solution of virtual number is proposed, the charging aspect is needed to be studied by SA5

## 4.6	Key Issue #6: application enablement aspects for A2P and P2A avatar communication

### 4.6.1	Description

Avatar communication is specified in normative Annex AC.11 of 3GPP TS 23.228 [4]. The avatar communication may be used in some A2P/P2A use cases, e.g. Avatar based customer service. In this case, when the user calls a brand, the avatar of the brand may serve this user, the user may also show his/her avatar to the brand. However, the avatar communication specified in normative Annex AC.11 of 3GPP TS 23.228 [4] is not supported in the A2P/P2A use cases.

In avatar communication, the avatar media rendering process may be performed by the sending UE, the receiving UE, or the network according to the rendering mode as defined in AC.11 in 3GPP TS 23.228 [4]. For example, in the UE centric avatar communication procedure, a P2A application data channel or a P2A2P application data channel is established and the avatar media rendering mode is set by the DC AS. In addition, what type of rendering mode (e.g., network centric with MF rendering, network centric with DC AS rendering) will be applied to the network-centric rendering is set by the DC AS. However, it is not clear how to determine the avatar media rendering mode for avatar communication in DC AS.

### 4.6.2	Open Issue

The open issues on the application enablement aspects for A2P and P2A avatar communication includes:

1.	Whether and how to enable the A2P and P2A avatar communication in MMTel Enabler layer.

2.	Whether and how the application to manage its Avatar related information (e.g. Avatar IDs and/or the associated information etc) .in the A2P/P2A avatar communication. Whether and how the Digital assets management specified in 3GPP TS 23.438 [8] can be used in this case.

3.	Whether and how to determine the avatar communication rendering modes in MMTEL Enabler layer.

## 4.x	Key Issue #X: <Key Issue Title>

### 4.x.1	Description

This clause provides a description of the key issue.

### 4.x.2	Open Issue

This clause provides the open issue(s) of the key issue.

# 5	Architectural requirements

## 5.1	General

This clause provides the generic architectural requirements identified.

## 5.2	Requirements

[AR-5.2-a] The MMTel Enablement service shall provide capability to support the data channel applications search on UE by providing data channel application search response to UE.

[AR-5.2-b] The MMTel Enablement service shall provide capability to support DC application and profile management in DCAR.

# 6	Solutions

## 6.1	Solution #1: DC Application list search

### 6.1.1	Description

This solution resolves Key Issue #2 "DC Application list search".

This solution uses the architecture of MMTel Enabler as specified 3GPP TS 23.392 [2].

### 6.1.2	Procedures

Figure 6.1.2-1 below illustrates the procedure of Getting DC Applications that fulfill requested search criteria from the MMTel Enabler Server.

Pre-conditions:

1.	The bootstrap data channels have been established as specified in 3GPP TS 23.228 [4].

Figure 6.1.2-1: MMTel Enabler Client in the UE search the DC application list

1.	The  MMTel Enabler Client sends a DC Application list search request to the MMTel Enabler Server. The request includes information elements as specified in Table 6.1,2-1.

Editor’s note:	Whether the URL for downloading the DC Application list, as specified in 3GPP TS 26.114 [5], can be modified (e.g., by adding some parameters) is FFS, and coordination with SA4 is required.

Table 6.1.2-1: Information elements in DC Application list search request

| Information element | Status | Description |
| --- | --- | --- |
| UE Information (see NOTE) | M | The UE related information required to identify the Root application (e.g. device type, device vendor, etc) |
| MMTel Enabler Client version (see NOTE) | M | The version of MMTel Enabler Client |
| Home operator | O | It indicates the home operator of the device and this shall be represented as <MCCMNC> combination. Whereby MCC and MNC shall be replaced by the respective values of the home network in decimal format and with a 2-digit MNC padded out to 3 digits by inserting a 0 at the beginning. |
| search criteria | O | The criteria included by the user in this request and used by the MMTel Enabler server to filter the DC Application list |
| NOTE: The MMTel service provider can provide the MMTel Enabler Client with different DC Applications based on this IE. |  |  |

2.	The MMTel Enabler server verifies that the sender is authorized to search the DC Application list and filter the DC Application list based on search criteria and other information included in the DC Application list search request. The MMTel Enabler Server may interact with DCSF for searching the DC Application list.

NOTE:	If DCSF is implemented together with  MMTel Enabler Server, then the DCSF may support the DC application profile functionality of MMTel Enabler Server function. The DCSF may also interact with the MMTel Enabler Server to support this procedure, i.e. The MMTel Enabler Client sends a request to the DCSF, which then sends the request to the MMTel Enabler Server. The response in the opposite direction is also sent to the MMTel Enabler Client through the DCSF.

3.	The MMTel Enabler Server sends a DC Application list search response to the MMTel Enabler client in the UE. The response message includes information elements as specified in clause 8.3.2.3.4 in 3GPP TS 23.392 [2].

### 6.1.3	Impacts on Existing Nodes and Functionality

In this solution, new parameters are proposed to be included in the DC Application list search request, which may impact the URL for downloading the DC Application list. Whether the URL for downloading the DC Application list, as specified in 3GPP TS 26.114 [5], can be modified (e.g., by adding some parameters) is FFS, and coordination with SA4 is required.

## 6.2	Solution #2: Northbound interface of DCAR

### 6.2.1	Description

This solution resolves Key Issue #4: Northbound interface of DCAR.

This solution uses the enhanced architecture of MMTel Enabler as specified 3GPP TS 23.392 [2]. The architecture is illustrated in Figure 6.2.1-1

Figure 6.2.1-1: MMTel Enabler architecture supports Northbound interface of DCAR

The interactions between the MMTel Enabler Server and DCAR are provided by the MMTel Enabler Server via MMTel-5 reference point. This reference point is the northbound interface of DCAR and supports:

-	the MMTel Enabler Server to manage the DC application and profiles needed in MMTel service on DCAR; and

-	the MMTel Enabler Server to get the DC application and profile from the DCAR.

### 6.2.2	Procedures

#### 6.2.2.1	Uploading DC application and profile to DCAR

Figure 6.2.2.1-1 below illustrates the procedure of uploading DC application and profile from the MMTel Enabler Server to DCAR. This procedure is a detailed procedure for the step 3 in the clause 8.2.2 of 3GPP TS 23.392 [2].

Pre-conditions:

1.	The DC application and profile have been configured to MMTel Enabler Server as specified in 3GPP TS 23.392 [2] by the MMTel service provider.

2.	The MMTel service provider decides to upload the DC application and profile to the DCAR so that they can be used by the MMTel service.

Figure 6.2.2.1-1: Uploading DC application and profile from the MMTel Enabler Server to DCAR

1.	The MMTel Enabler Server sends a DC application and profile upload request to the DCAR. The request message includes information elements as specified in Table 6.2.2.1-1.

Table 6.2.2.1-1: Information elements in DC application and profile upload request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the DC Application provider (VAL service provider) performing the request. |
| MMTel Enabler Server ID | M | The identity of the MMTel Enabler Server |
| DC Application list | M | A list of DC application and configuration parameters included in this request. Each element in this list contains a Data Channel Application package and corresponding configuration parameters. The detailed information elements of DC application list are listed in the Table 8.2.2.3.1-2 of 3GPP TS 23.392 [2]. |

2.	Upon receiving the request, if required by the mmtel service provider, the DCAR validates if the requester is authorized for the request based on the MMTel Enabler Server ID. Then the DCAR stores the DC application and the related profile.

3.	The DCAR sends a DC application and profile upload response to the MMTel Enabler Server. The response message includes information elements as specified in Table 6.2.2.1-2.

Table 6.2.2-2: Information elements for DC application and profile upload response

| Information element | Status | Description |
| --- | --- | --- |
| DC application and profile configuration response list | M | A list of DC application and profile configuration response. This list contains a response for each DC Application. The detailed information is listed in Table 8.2.2.3.2-2 of 3GPP TS 23.392 [2]. |

#### 6.2.2.2	Updating DC application and profile to DCAR

Figure 6.2.2.2-1 below illustrates the procedure of uploading DC application and profile from the MMTel Enabler Server to DCAR. This procedure is a detailed procedure for the step 3 in the clause 8.2.3 of 3GPP TS 23.392 [2].

Pre-conditions:

1.	The DC application and profile have been configured to DCAR as specified in clause 6.2.2.1.

2.	The MMTel service provider decides to update the DC application and profile to the DCAR so that they can be used by the MMTel service.

Figure 6.2.2.2-1: Updating DC application and profile from the MMTel Enabler Server to DCAR

1.	The MMTel Enabler Server sends a DC application and profile update request to the DCAR. The request message includes information elements as specified in Table 6.2.2.2-1.

Table 6.2.2.2-1: Information elements in DC application and profile update request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the DC Application provider (VAL service provider) performing the request. |
| MMTel Enabler Server ID | M | The identity of the MMTel Enabler Server |
| DC Application list | M | A list of DC application and configuration parameters included in this request. Each element in this list contains a Data Channel Application package and corresponding configuration parameters. The detailed information elements of DC application list are listed in the Table 8.2.2.3.1-2 of 3GPP TS 23.392 [2]. |

2.	Upon receiving the request, if required by the mmtel service provider, the DCAR validates if the requester is authorized for the request based on the MMTel Enabler Server ID. Then the DCAR updates the DC Application or DC application profile for each DC Application based on information provided in the request.

3.	The DCAR sends a DC application and profile update response to the MMTel Enabler Server. The response message includes information elements as specified in Table 6.2,2.2-2.

Table 6.2.2.2-2: Information elements for DC application and profile update response

| Information element | Status | Description |
| --- | --- | --- |
| DC application and profile update response list | M | A list of DC application and profile update response. This list contains a response for each DC Application. The detailed information is listed in Table 8.2.2.3.2-2 of 3GPP TS 23.392 [2]. |

#### 6.2.2.3	Deleting DC application and profile from DCAR

Figure 6.2.2.3-1 below illustrates the procedure of deleting DC application and profile from  DCAR. This procedure is a detailed procedure for the step 3 in the clause 8.2.4 of 3GPP TS 23.392 [2].

Pre-conditions:

1.	The DC application and profile have been configured to DCAR as specified in clause 6.2.2.1.

2.	The MMTel service provider decides to delete the DC application and profile to the DCAR so that they can be used by the MMTel service.

Figure 6.2.2.3-1: Deleting DC application and profile from DCAR

1.	The MMTel Enabler Server sends a DC application delete request to the DCAR. The request message includes information elements as specified in Table 6.2.2.3-1.

Table 6.2.2.3-1: Information elements in DC application delete request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the DC Application provider (VAL service provider) performing the request. |
| MMTel Enabler Server ID | M | The identity of the MMTel Enabler Server |
| List of APPID | M | List of identifiers of the DC application to be deleted. |

2.	Upon receiving the request, if required by the mmtel service provider, the DCAR validates if the requester is authorized for the request based on the MMTel Enabler Server ID. Then the DCAR deletes the DC application and profile (DC applications and profile need to be deleted at the same time) and deletes the mapping of APPID and DC application profile.

3.	The DCAR sends a DC application delete response to the MMTel Enabler Server. The response message includes information elements as specified in Table 6.2,2.3-2.

Table 6.2.2.3-2: Information elements for DC application delete response

| Information element | Status | Description |
| --- | --- | --- |
| DC Application delete response list | M | A list of Application deletion response. The detailed information is listed in Table 6.2.2.3-3. |

Table 6.2.2.3-3: Information elements in DC Application delete response list

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the deletion of the DC Application was successful or failure. |
| > Cause | O | Indicates the cause of failure. |

### 6.2.3	Impacts on Existing Nodes and Functionality

In this solution, a new reference point MMTel-5 between the MMTel Enabler Server and DCAR is introduced to the architecture of MMTel Enabler. The interaction  via MMTel-5 reference point in this solution is needed to be supported by both MMTel Enabler Server and DCAR.

## 6.3	Solution #3: Alignment 8.4.2 and 8.4.3 service flows with SA2 Rel-19 support for IMS Capability Exposure Framework

### 6.3.1	Description

This solution resolves KI#1: Alignment eMMTel service flows with SA2 Rel-19 support for IMS Capability Exposure Framework. This solution addresses the  procedures in clause 8.4.2 and 8.4.3 of 3GPP TS 23.392 [2] in Rel-20.

### 6.3.2	Procedures

#### 6.3.2.1	Update to existing procedures in 8.4.2

The following changes are proposed to clause 8.4.2. (New text is highlighted in bold italics, and removed text is marked with a remove mark.)

Editor’s note: Whether this procedure needs to be further updated depends on whether more IMS session management capabilities will be provided by the IMS/Core Network. This is FFS.

### 8.4.2	Application calling service API with Data Channel capability

#### 8.4.2.1	General

The MMTel Enabler Server exposes the Application calling service API with Data Channel capability to the Applications in order to support establishing a call between Application and DCMTSI client.

#### 8.4.2.2	Procedure

##### 8.4.2.2.1	Establishing an Application Call with Data Channel capability between non-IMS application and DCMTSI client

Figure 8.4.2.2.1-1 illustrates the operation to establish an Application Call with Data Channel capability between non-IMS application and DCMTSI client.

Pre-conditions:

1.	The Application is authorized to use Application calling service API provided by the MMTel Enabler Server.

2.	The MMTel Enabler Server is authorized to use IMS DC capability APIs as defined in 3GPP TS 23.228 [3] or authorized to use the NEF exposed IMS session management capabilities as specified in 3GPP TS 23.502 [12].

3.	The MMTel Enabler Server is authorized to use OMA ThirdPartyCall API as specified in OMA-TS-REST_NetAPI_ThirdPartyCall [7].

Figure 8.4.2.2.1-1: Establishing a Application Call with Data Channel capability between non-IMS applications and DCMTSI client

1.	The Application sends an Application Call with DC capability establishment request to the MMTel Enabler Server. The request message includes information elements as specified in clause 8.4.2.3.1.

2.	Upon receiving the request, the MMTel Enabler Server validates if the requester is authorized for the request, if the requester is authorized then the MMTel Enabler Server can utilize the OMA Third Party Call API [7] to establish a Call session with audio and/or video media as specified in OMA-TS-REST_NetAPI_ThirdPartyCall and obtain the result of the Third-Party Call establishment. The MMTel Enabler Server may also obtains the IMS Session ID associated with the Third Party Call as specified in RFC 7989 [10], and handles the mapping between the OMA call session ID and the IMS session ID.

NOTE 1:	The OMA call session ID attribute in the OMA Third Party Call API is generated by the entity implementing the OMA APIs, which is different from the IMS session ID.

3,	The MMTel Enabler Server utilizes the IMS session management capabilities provided by IMS AS as specified in Annex AA.2.4 of  3GPP TS 23.228 [4] or IMS session management capabilities provided by NEF as specified in clause 5.2.6.40 of 3GPP TS 23.502 [12]DC update capability of the IMS core network to add DC media to the existing Application Call Session as specified in Annex AG.2.1AC of 3GPP TS 23.228 [3]. The IMS core network returns the update result.

3.	The MMTel Enabler Server sends an Application Call establishment response to the Application. The response message includes information elements as specified in clause 8.4.2.3.2.

NOTE 2:	In this release, the MMTel Enabler Server is deployed in the PLMN operator domain.

#### 8.4.2.3	Information flows

##### 8.4.2.3.1	Application Call establishment request

The information elements of Application Call establishment request are as same as the elements in Table 8.4.3.3.1-1.

##### 8.4.2.3.2	Application Call establishment response

The information elements of Application Call establishment response are as same as the elements in Table 8.4.3.3.2-1.

#### 6.3.2.2	Update to existing procedures in 8.4.3

The following changes are proposed to clause 8.4.3. (New text is highlighted in bold italics, and removed text is marked with a remove mark.)

Editor’s note: Whether this procedure needs to be further updated depends on whether more IMS session management capabilities will be provided by the IMS/Core Network. This is FFS.

### 8.4.3	Third-Party Call service API with Data Channel capability

#### 8.4.3.1	General

The MMTel Enabler Server exposes the Third-Party Call service API with Data Channel capability to the Vertical service provider in order to support establishing a call between two UEs.

#### 8.4.3.2	Procedure

##### 8.4.3.2.1	Establishing a Third-Party Call with Data Channel capability

Figure 8.4.3.2.1-1 illustrates the operation to establish a Third-Party Call with Data Channel capability.

Pre-conditions:

1.	The Vertical service provider is authorized to use Third-Party Call service API provided by the MMTel Enabler Server.

2.	The MMTel Enabler Server is authorized to use the IMS DC capability APIs as defined in 3GPP TS 23.228 [3] or authorized to use the NEF exposed IMS session management capabilities as specified in 3GPP TS 23.502 [12].

3.	The MMTel Enabler Server is authorized to use OMA THIRD PARTY CALL API as specified in OMA-TS-REST_NetAPI_ThirdPartyCall [7].

Figure 8.4.3.2.1-1: Establishing a Third-Party Call with Data Channel capability

1.	The Vertical service provider sends a Third-Party Call with DC capability establishment request to the MMTel Enabler Server. The request message includes information elements as specified in clause 8.4.3.3.1.

2.	Upon receiving the request, the MMTel Enabler Server validates if the requester is authorized for the request, If the requester is authorized then the MMTel Enabler Server utilizes the OMA Third Party Call API [7] to establish a Third-Party Call session with audio and/or video media as specified in OMA-TS-REST_NetAPI_ThirdPartyCall and obtain the result of the Third Party Call establishment. The MMTel Enabler Server obtains IMS Session ID associated with the Third Party Call as specified in RFC 7989 [10],the OMA API call Session ID and handles the mapping between OMA call session ID and the IMS session ID

NOTE 1:	The OMA call session ID attribute in the OMA Third Party Call API is generated by the entity implementing the OMA APIs, which is different from the IMS session Id.

3.	the MMTel Enabler Server utilizes the IMS session management capabilities provided by IMS AS as specified in Annex AA.2.4 of 3GPP TS 23.228 [4] or IMS session management capabilities provided by NEF as specified in clause 5.2.6.40 of 3GPP TS 23.502 [12]DC update capability of the IMS core network to add DC media to the existing Third-Party Call Session as specified in Annex AG.2.1 of 3GPP TS 23.228 [3], and the IMS core network returns the update result.

4.	The MMTel Enabler Server sends a Third-Party Call with DC capability establishment response to the Vertical service provider. The response message includes information elements as specified in clause 8.4.3.3.2.

NOTE 2:	In this release, the MMTel Enabler Server is deployed in the PLMN operator domain.

#### 8.4.3.3	Information flows

##### 8.4.3.3.1	Third-Party Call with DC capability establishment request

Table 8.4.3.3.1-1 describes information elements for the Third-Party Call with DC capability establishment request from the Application Server to the MMTel Enabler Server.

Table 8.4.3.3.1-1: Third-Party Call with DC capability establishment request

| Information element | Status | Description | Related OMA API Element |
| --- | --- | --- | --- |
| Originating ID (see NOTE) | M | The identifier of the caller. | The first element in the participant list, which considered to denote the “A-Party” specified in OMA Third Party Call API [7]. |
| Terminating ID (see NOTE) | M | The identifier of the callee. | The second element in the participant list, which considered to denote the “B-Party” specified in OMA Third Party Call API [7]. |
| Media information | O | Identifier of one or more media type(s) which is expected to be used in the call, i.e. the media type(s) to be applied to the participants in the call session. In this IE, the media type(s) includes a) audio; b) video; c) audio and video. The detailed media information, e.g. the information included in the SDP of MMTel call session is negotiated by the underlying network. If the parameter is absent, the media type(s) and detailed media information are all negotiated by the underlying network. | The mediaInfo of the CallSessionInformation specified in OMA Third Party Call API [7]. |
| DC media information | O | Identifier of whether DC media is expected to be used in this call | N/A |
| Application Profile requested | O | The DC application profile expected to be used in this call | N/A |
| Notification information | M | The address where the call related notification, e.g. whether the call between UE A and UE B is established successfully or not, is sent to. | The callbackReference of the CallSessionInformation specified in OMA Third Party Call API [7]. |
| NOTE: The Originating ID and Terminating ID may or may not be MSISDN based identifier, i.e. a private identifier in the Application domain. If Originating ID or Terminating ID is not MSISDN based identifier, the MMTel Enabler Server will translate it into a routable ID in the IMS domain and record the mapping relationship between the Originating ID/Terminating ID and the routable ID. |  |  |  |

##### 8.4.3.3.2	Third-Party Call with DC capability establishment response

Table 8.4.3.3.2-1 describes information elements for the Third-Party Call with DC capability establishment response from the Application Server to the MMTel Enabler Server.

Table 8.4.3.3.2-1: Third-Party Call with DC capability establishment response

| Information element | Status | Description | Related OMA API Element |
| --- | --- | --- | --- |
| Call establishment result | M | Indication if the Call establishment is success or failure. | The HTTP response codes of the created call session as specified in OMA REST_NetAPI_Common [11]. |
| Session ID (see NOTE 1) | O | The identifier of the call session. | The callSessionId as specified in OMA Third Party Call API [7]. |
| Failure Cause (see NOTE 2) | O | The reason for failure | The Policy Exception and Service Exception fault as specified in OMA Third Party Call API [7]. |
| Originating ID (see NOTE 3) | M | The identifier of the caller. | The first element in the participant list, which considered to denote the “A-Party” specified in OMA Third Party Call API [7]. |
| Terminating ID (see NOTE 3) | M | The identifier of the callee. | The second element in the participant list, which considered to denote the “B-Party” specified in OMA Third Party Call API [7]. |
| Media information | O | Identifier of one or more media type(s) which is expected to be used in the call, i.e. the media type(s) to be applied to the participants in the call session. In this IE, the media type(s) includes a) audio; b) video; c) audio and video. The detailed media information, e.g. the information included in the SDP of MMTel call session is negotiated by the underlying network. If the parameter is absent, the media type(s) and detailed media information are all negotiated by the underlying network. | The mediaInfo of the CallSessionInformation specified in OMA Third Party Call API [7]. |
| DC media information | O | Identifier of whether DC media is expected to be used in this call. | N/A |
| Application Profile requested | O | The DC application profile expected to be used in this call. | N/A |
| NOTE 1: This IE shall only be present when the Call establishment result is Success. NOTE 2: This IE shall only be present when the Call establishment result is Failure. NOTE 3: The Originating ID and Terminating ID may or may not be MSISDN based identifier, i.e. a private identifier in the Application domain. If Originating ID or Terminating ID is not MSISDN based identifier, the MMTel Enabler Server will translate it into a routable ID in the IMS domain and record the mapping relationship between the Originating ID/Terminating ID and the routable ID. |  |  |  |

### 6.3.3	Impacts on Existing Nodes and Functionality

In this solution, the MMTel Enabler server needs to utilize new IMS session management capabilities provided by the NEF as specified in clause 5.2.6.40 of 3GPP TS 23.502 [12], and also needs to utilize the procedures specified in Annex AG.2.1 of 3GPP TS 23.228 [3] instead of Annex AC of 3GPP TS 23.228 [3].

## 6.4	Solution #4: Enable the A2P avatar communication

### 6.4.1	Description

This solution resolves Open issue#1 of KI#5 on application enablement aspects for A2P avatar communication.

This solution uses the architecture of MMTel Enabler as specified 3GPP TS 23.392 [2].

### 6.4.2	Procedures

Figure 6.4.2-1 below illustrates the procedure of enablement of the A2P avatar communication where AS performs avatar animation. This procedure can be used by an application, e.g., a brand's customer service center, to initiate an avatar call with a user and show the brand's avatar to them.

Pre-conditions:

1.	The Application Server is authorized to use APIs provided by the MMTel Enabler Server.

2.	The MMTel Enabler Server is authorized to use IMS DC capability APIs as defined in 3GPP TS 23.228 [4] or authorized to use the NEF exposed IMS session management capabilities as specified in 3GPP TS 23.502 [3].

.

Figure 6.4.2-1: Enablement of the A2P avatar communication-AS performs avatar animation

1.	The Application server establishes an application call with Data Channel capability with UE as specified in clause 8.4.2 of 3GPP TS 23.392 [2].

2.	If the Avatar Representation and the associated information are stored in BAR as specified in 3GPP TS 23.228 [4], the Application server may retrieve the Avatar Representation and the associated information from BAR.

3.	The Application server performs avatar animation.

4.	The Application server transmits the animated video stream over RTP to the UE via the IMS session as specified in clause AC.11.3.2.1 of 3GPP TS 23.228 [4]. In this procedure, the Application server acts as the Sending UE and utilizes the step 7 and step 8 of the Sending UE centric procedure. The IMS session is established in step 1.

### 6.4.3	Impacts on Existing Nodes and Functionality

In this solution, the Application server needs to utilize the procedure specified in clause 8.4.2 of 3GPP TS 23.392 [2] to establishes an application call with Data Channel capability with UE No additional requirement applies to MMTel Enabler Server in this solution.

The Application server also needs to support avatar animation and transmits the animated video stream over RTP to the UE via the IMS session as specified in clause AC.11.3.2.1 of 3GPP TS 23.228 [4]. In this procedure, the Application server acts as the Sending UE and utilizes the step 7 and step 8 of the Sending UE centric procedure.

## 6.5	Solution #5: Enablement of standalone DC

### 6.5.1	Description

This solution resolves Key Issue #3: Enablement of standalone DC.

This solution uses the architecture of MMTel Enabler as specified 3GPP TS 23.392 [2].

### 6.5.2	Procedures

Figure 6.5.2-1 below illustrates the procedure of Enablement of standalone DC. This procedure provides applications with direct access to Standalone Data Channel capabilities for application-layer interactions (e.g., menu content exchange), abstracting away the underlying IMS session awareness requirements.

Pre-conditions:

1.	The Application Server is authorized to use APIs provided by the MMTel Enabler Server.

2.	The MMTel Enabler Server is authorized to use IMS DC capability APIs as defined in 3GPP TS 23.228 [4] or authorized to use the NEF exposed IMS session management capabilities as specified in 3GPP TS 23.502 [3].

.

Figure 6.5.2-1: Enablement of standalone DC

1.	The Application server sends a Send application message request to the MMTel Enabler client. The request carries an application payload destined for the DC application on the UE, which is its client counterpart. The request message includes information elements as specified in Table 6.5,2-1.

Table 6.5.2-1: Information elements in Send application message request

| Information element | Status | Description |
| --- | --- | --- |
| Originating ID (see NOTE) | M | The identifier of the Application server. |
| Terminating ID (see NOTE) | M | The identifier of the callee. |
| Message ID | M | The unique identifier of this application message |
| application payload | M | The application specific payload destined for the DC application on the UE |
| NOTE: The Originating ID and Terminating ID may or may not be MSISDN based identifier, i.e. a private identifier in the Application domain. If Originating ID or Terminating ID is not MSISDN based identifier, the MMTel Enabler Server will translate it into a routable ID in the IMS domain and record the mapping relationship between the Originating ID/Terminating ID and the routable ID. |  |  |

2.	Upon receiving the request, the MMTel Enabler server checks whether the MMTel session with Data Channel capability has been established between the Application server by using the procedure specified in clause 8.4.2 of 3GPP TS 23.392 [2]. If the MMTel session with Data Channel capability has been established, the MMTel Enabler server can send the Send application message request to the UE directly through the Data Channel and step 3 to step 6 can be skipped.

3.	The MMTel Enabler server caches the Send application message request until the standalone DC has been established in step 6.

4.	The MMTel Enabler server utilizes the IMS session management capabilities provided by IMS AS as specified in Annex AA.2.4 of  3GPP TS 23.228 [4] or IMS session management capabilities provided by NEF as specified in clause 5.2.6.40 of 3GPP TS 23.502 [3] to establish the standalone DC between MMTel Enabler server and UE. In this procedure, the MMTel Enabler server utilizes the procedure specified in Annex AG.2.2.2 of 3GPP TS 23.228 [4].

5.	the IMS handles the establishment of standalone DC between MMTel Enabler server and UE as specified in Annex AG.2.2.2 of  3GPP TS 23.228 [4].

6.	The standalone Application Data Channel is established between MMTel Enabler server and UE.

7.	The MMTel Enabler server sends the cached Send application message request to the MMTel Enabler client in the UE via the Application Data Channel. The MMTel Enabler client sends the application payload to the corresponding DC application in the UE.

NOTE:	How the MMTel Enabler client sends the application payload to the corresponding DC application in the UE is internal in the UE and implementation specific.

8.	The MMTel Enabler client sends the Send application message response to the MMTel Enabler server. The response message includes information elements as specified in Table 6.5,2-2.

Table 6.5.2-1: Information elements in Send application message response

| Information element | Status | Description |
| --- | --- | --- |
| Originating ID (see NOTE 1) | M | The identifier of the Application server. |
| Terminating ID (see NOTE 1) | M | The identifier of the callee. |
| Message ID | M | Identifier of the Request that is being responded to. |
| Result | M | Indicates if the sending of this application message is success or failure. |
| Reason (see NOTE 2) | O | The reason for failure |
| NOTE 1: The Originating ID and Terminating ID may or may not be MSISDN based identifier, i.e. a private identifier in the Application domain. If Originating ID or Terminating ID is not MSISDN based identifier, the MMTel Enabler Server will translate it into a routable ID in the IMS domain and record the mapping relationship between the Originating ID/Terminating ID and the routable ID. NOTE 2: This IE shall only be present when the sending result is failure. |  |  |

9.	The MMTel Enabler server sends the Send application message response to the Application server.

10.	Subsequent application interactions can take place over the established standalone application Data Channel.

### 6.1.3	Impacts on Existing Nodes and Functionality

In this solution, the MMTel Enabler server needs to utilize the procedure specified in Annex AG.2.2.2 of  3GPP TS 23.228 [4].

## 6.X	Solution #X: <Solution Title>

### 6.X.1	Description

This clause will describe the solution principles and architecture assumptions for corresponding key issue(s). (Sub) clause(s) may be added to capture details.

### 6.X.2	Procedures

This clause describes high-level procedures and information flows for the solution.

### 6.X.3	Impacts on Existing Nodes and Functionality

This clause captures impacts on existing 3GPP nodes and functional elements.

# 7	Overall Evaluation

## 7.1	General

This clause will provide the  mapping between KIs and solutions. Evaluation of different solutions. will be also captured in this clause.

## 7.2	Mapping of Solutions to Key Issues

Table 7.2-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 | #5 | #6 |
| #1 |  | X |  |  |  |  |
| #2 |  |  |  | X |  |  |

## 7.3	Solution evaluation

# 8	Conclusions

This clause will list conclusions that have been agreed during the  study item activities.

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-03 | SA6#66 |  | - | - | - | Proposed skeleton approved at S6#66 | 0.0.0 |
| 2025-04 | SA6#66 |  |  |  |  | Implemented pCRs approved in S6#66: S6-251200, S6-251203, S6-251362, S6-251363, S6-251364, S6-251217 Editorial changes by the rapporteur | 0.1.0 |
| 2025-05 | SA6#67 |  |  |  |  | Implemented pCRs approved in S6#67: S6-252198, S6-252423 Editorial changes by the rapporteur | 0.2.0 |
| 2025-09 | SA6#68 |  |  |  |  | Implemented pCRs approved in S6#68: S6-253665, S6-253664, S6-253338 Editorial changes by the rapporteur | 0.3.0 |
| 2025-10 | SA6#69 |  |  |  |  | Implemented pCRs approved in S6#69: S6-254626, S6-254627, S6-254628, S6-254299, S6-254629, S6-254630, S6-254631, S6-254305 Editorial changes by the rapporteur,including fix the formatting issue of the figures | 0.4.0 |