| 3GPP TR 23.700-59 V0.3.0 (2024-04) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on architecture enhancements of UAS, UAV and UAM; Phase 3 (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	9

3.1	Terms	9

3.2	Symbols	9

3.3	Abbreviations	9

4	Architectural Assumptions and Requirements	9

4.1	Architectural Assumptions	9

4.2	Architectural Requirements	10

5	Key Issues	10

5.1	Key Issue #1: Enhancement of NEF services to support service exposure and interactions between MNOs and UTM functions	10

5.1.1	Description	10

5.2	Key Issue #2: Network-assisted/ground-based mechanism for DAA (Detect And Avoid) with 5GS information	10

5.2.1	Description	10

5.3	Key Issue #3: Support of No Transmit Zones	11

5.3.1	Description	11

6	Solutions	12

6.0	Mapping of Solutions to Key Issues	12

6.1	Solution #1: Support Pre-Mission Planning and In-Mission Monitoring Flight	12

6.1.1	Key Issue mapping	12

6.1.2	Description	12

6.1.3	Procedures and Parameters	13

6.1.3.1	Procedure for NEF Assist Pre-mission Flight Planning	13

6.1.3.2	Procedure for NEF Assist In-mission Flight Monitoring	15

6.1.3.3	Parameters in Request for Movement Behaviour Analytics	16

6.1.4	Impacts on services, entities and interfaces	16

6.2	Solution #2: UAV flight path deviation exposure	17

6.2.1	Key Issue mapping	17

6.2.2	Description	17

6.2.3	Procedures	17

6.2.3.1	Monitoring & reporting of UAV flight path deviation	17

6.2.4	Impacts on services, entities and interfaces	19

6.3	Solution #3: UAV flight planning and monitoring	19

6.3.1	Key Issue mapping	19

6.3.2	Description	19

6.3.3	Procedures	20

6.3.4	Impacts on services, entities and interfaces	20

6.4	Solution #4: Network-supported Tactical Deconfliction	20

6.4.1	Key Issue mapping	20

6.4.2	Description	21

6.4.2.1	Introduction	21

6.4.2.2	Solution Overview	21

6.4.2.3	Solution Architecture	22

6.4.3	Procedures	22

6.4.3.1	Overall Information Flow	22

6.4.3.2	UAV-LDS Connectivity Establishment for LDS@Edge	24

6.4.4	Impacts on services, entities and interfaces	25

6.5	Solution #5: Support Network-assisted DAA with Existing 5GC Services	25

6.5.1	Key Issue mapping	25

6.5.2	Description	25

6.5.3	Procedures and Parameters	27

6.5.3.1	Procedure for UAV/UAV-C Triggered Network-assisted DAA	27

6.5.3.2	Procedure for AAM Triggered Network-assisted DAA	29

6.5.3.3	Parameters in Request for Relative Proximity Analytics	29

6.5.4	Impacts on services, entities and interfaces	30

6.6	Solution #6: UTM requests information used for DAA	30

6.6.1	Key Issue mapping	30

6.6.2	Description	30

6.6.3	Procedures	31

6.6.4	Impacts on services, entities and interfaces	31

6.7	Solution #7: Mobility Enhancements for enforcements of NTZ	32

6.7.1	Key Issue mapping	32

6.7.2	Description	32

6.7.2.1	Introduction	32

6.7.2.2	Solution Overview	32

6.7.2.3	Extension of Service Restrictions for NTZs	33

6.7.2.4	NTZ Restriction Policies	34

6.7.2.5	Per-cell indication of the presence of NTZs:	34

6.7.2.6	Solution Architecture	35

6.7.3	Procedures	35

6.7.3.0	General	35

6.7.3.1	Extension of Service Restrictions for NTZs	35

6.7.3.2	NTZ Restriction Policies	36

6.7.4	Impacts on services, entities and interfaces	37

6.8	Solution #8: Network support for NTZ management	37

6.8.1	Key Issue mapping	37

6.8.2	Description	37

6.8.3	Procedures	38

6.8.4	Impacts on Existing Nodes and Functionality	39

6.9	Solution #9: Enabling NTZ support for aerial UEs	39

6.9.1	Key Issue mapping	39

6.9.2	Description	39

6.9.3	Procedures	40

6.9.4	Impacts on services, entities and interfaces	45

6.10	Solution #10: Enabling 5GC support for multiple USS serving different geographical areas	46

6.10.1	Key Issue mapping	46

6.10.2	Description	46

6.10.3	Procedures	48

6.10.4	Impacts on services, entities and interfaces	52

6.11	Solution #11: Multi-USS Support in 5GC	52

6.11.1	Key Issue mapping	52

6.11.2	Description	52

6.11.3	Procedures	53

6.11.4	Impacts on services, entities and interfaces	54

6.12	Solution #12: Support redundant connections for C2 communication reliability	54

6.12.1	Key Issue mapping	54

6.12.2	Description	54

6.12.3	Procedures	55

6.12.3.1 UE initiates PDU Session establishment for 2 redundant C2 communications with a UAV-C	55

6.12.4	Impacts on services, entities and interfaces	56

6.13	Solution #13: Enhancement of NEF services for C2 communication assistance information	56

6.13.1	Key Issue mapping	56

6.13.2	Description	56

6.13.3	Procedures	57

6.13.4	Impacts on services, entities and interfaces	58

6.14	Solution #14: C2 Communication Reliability with Redundant User Plane Paths	58

6.14.1	Description	58

6.14.2	Procedures	59

6.14.3	Impacts on services, entities and interfaces	62

6.15	Solution #15: NTZ restriction Provisioning aspects	62

6.15.1	Key Issue mapping	62

6.15.2	Description	62

6.15.3	Procedures	62

6.15.3.1	NTZ Provisioning	62

6.15.3.2	UE Configuration Update procedure for transparent UE Policy delivery	64

6.15.3.3 No-transmission zone parameters & UE behaviour	64

6.15.4	Impacts on services, entities and interfaces	64

6.16	Solution #16: NTZ Enforcement in 5GC	65

6.16.1	Key Issue mapping	65

6.16.2	Description	65

6.16.3	Procedures	65

6.16.4	Impacts on services, entities and interfaces	67

6.17	Solution #17: Support of No Transmit Zone	67

6.17.1	Key Issue mapping	67

6.17.2	Description	67

6.17.3	Procedures	68

6.17.4	Impacts on services, entities and interfaces	68

6.X	Solution #X: <Solution Title>	69

6.X.1	Key Issue mapping	69

6.X.2	Description	69

6.X.3	Procedures	69

6.X.4	Impacts on services, entities and interfaces	69

7	Overall Evaluation	69

8	Conclusions	70

Annex A: Background Information about No Transmit Zones	71

A.1	CEPT Decision 22(07)	71

Annex B: Change history	73

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document is to investigate and identify potential architecture and system level enhancements to support additional scenarios and requirements for UAV (Uncrewed Aerial Vehicle) and UAM (Urban Air Mobility) including:

-	Enhancement of NEF services to support service exposure and interactions between MNOs and UTM functions for i.e. pre-mission flight planning, in-mission flight monitoring, C2 communication reliability, interfacing with UTM (e.g. supporting the scenario of multiple USS serving the geographical areas corresponding to UAV flight path).

-	Support of network-assisted/ground-based mechanism for DAA (Detect And Avoid).

-	Support of no-transmit zones for UAVs.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.256: "Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking; Stage 2".

[3]	ECC Decision (22)07 (cept.org) https://docdb.cept.org/download/4240: "Harmonised technical conditions for the usage of aerial UE for communications based on LTE and 5G NR in the bands 703-733 MHz, 832-862 MHz, 880-915 MHz, 1710- 1785 MHz, 1920-1980 MHz, 2500-2570 MHz and 2570- 2620 MHz harmonised for MFCN".

[4]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[5]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[6]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[7]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[8]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[9]	3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage-2".

[10]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

No-transmit zones: Geographical area where aerial UE are not allowed to operate in a certain frequency band. The purpose and requirements of NTZ is described in ECC Decision (22)07 [3].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

DAA	Detect and Avoid

LDS	Localized DAA Server

NTZ	No-Transmit Zone

NWDAA	Network-Based/Assisted DAA

RTA	Restricted Transmission Area

# 4	Architectural Assumptions and Requirements

## 4.1	Architectural Assumptions

The following architectural assumptions apply:

-	For solutions to enable network-assisted/ground-based mechanism for DAA (Detect And Avoid),

-	co-existence with and leveraging, to the extent possible, Direct DAA solutions specified in TS 23.256 [2] shall be considered.

-	sensing related information is out of scope of this study.

-	Regarding C2 communication reliability aspects in KI#1, only C2 over the Uu interface is considered in this study, and UAV using multiple-PLMN connectivity to support C2 communication reliability will not be considered in this release.

The following assumptions apply to the support of NTZ:

-	NTZ may be defined according to regional/national requirements;

-	an NTZ may map to one or more cells or a fraction of a cell, or overlap different cells in a mobile operator network.

NOTE:	NTZ scope and mapping to cell(s) need to be coordinated with RAN WGs.

-	Replanning of existing tracking areas and cells based on the presence of NTZs is not assumed.

## 4.2	Architectural Requirements

Editor's note:	This clause provides list of architectural requirements, if needed.

-	The existing procedures specified in TS 23.256 [2] should be reused as much as possible for solutions.

-	Service exposure mechanisms defined in TS 23.256 [2] should be reused as much as possible.

The following architectural requirements apply to support NTZ:

-	No transmissions in the restricted frequency bands are allowed from an aerial UE located in the NTZ, regardless of the service type, according to local regulations. Aerial UEs supporting NTZ are configured with NTZ information.

NOTE:	What information consists the NTZ information is based on the conclusion of key issue#3.

-	Reception of downlink data from the network by the aerial UE located in the NTZ is allowed given the regulatory requirements are not violated.

# 5	Key Issues

## 5.1	Key Issue #1: Enhancement of NEF services to support service exposure and interactions between MNOs and UTM functions

### 5.1.1	Description

In this key issue, the following aspects are required to be studied:

-	whether and how to enhance NEF services to support service exposure and interactions between MNOs and UTM functions for supporting i.e.

-	Pre-mission flight planning and in-mission flight monitoring for UAVs.

-	C2 communication reliability.

-	The scenario of multiple USS serving different geographical areas corresponding to the UAV flight path.

NOTE:	In the scope of this key issue, UTM can represent any authorized aviation AF that may require interaction with the MNO for the functions listed above.

## 5.2	Key Issue #2: Network-assisted/ground-based mechanism for DAA (Detect And Avoid) with 5GS information

### 5.2.1	Description

Network-assisted/ground-based mechanism for DAA (NWDAA) for tactical deconfliction, collision avoidance, and UTM control of UAV flight path, can be considered a complement for existing DAA based on the PC5 reference point specified in Rel-18.

In this key issue, the following aspects are required to be studied:

-	Study whether and how to enable network-assisted/ground-based mechanism for DAA (Detect And Avoid):

-	Any architectural impacts for the support of NWDAA.

-	Whether and what information is needed for NWDAA:

-	Study which existing information collected and generated in the 5GS can be utilised to enable NWDAA.

-	Study whether any and what type of new information may be collected and/or generated in the 5GS to support NWDAA.

-	Whether and how to provide UTM and UAVs with the information collected or generated by the 5G system for the purpose of NWDAA.

## 5.3	Key Issue #3: Support of No Transmit Zones

### 5.3.1	Description

This key issue relates to the introduction by CEPT ECC Decision 22(07) [3] of No Transmit Zones for aerial UEs. The ECC Decision asserts that a mechanism is necessary to ensure that aerial UEs respect no-transmit zones in order to protect incumbent radio systems from potential interference from aerial UEs.

Since the ECC Decision does not identify any specific RAT, NTZs can be supported by both LTE and NR.

This key issue addresses the following aspects:

-	How to ensure an aerial UE respects no-transmit zones, including:

-	whether a mobile network cells overlapping completely or partially with the NTZ and using the restricted frequency bands of the NTZ;

-	whether mechanisms are needed to differentiate aerial UEs that support functions defined for NTZs in Rel. 19 and aerial UEs that don't;

-	what if any, specific aerial UE behaviour when the aerial UE approaches, enters, or exits the NTZ.

-	Whether and how to enable configuration of NTZ information in the aerial UE.

-	Whether to allow the enforcement of no-transmit zone(s) for both aerial UEs in connected mode and aerial UEs in idle mode and if yes then how.

NOTE:	Any potential solutions developed shall be coordinated with RAN WGs or progressed together with RAN WGs input.

For aerial UEs supporting NTZ regulations, it is assumed that they are configured with NTZ information, so that these aerial UEs will be aware of the presence of NTZs.

Editor's note:	What is requirement to 3GPP system with regard to aerial UEs not supporting 3GPP Rel-19 NTZ mechanism and how to handle these aerial UEs need to be addressed during the study and conclusion of KI#3 and coordination with RAN WGs.

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions |  |  |  |
| --- | --- | --- | --- |
|  | <Key Issue #1> | <Key Issue #2> | <Key Issue #3> |
| #1 | X |  |  |
| #2 | X |  |  |
| #3 | X |  |  |
| #4 |  | X |  |
| #5 |  | X |  |
| #6 |  | X |  |
| #7 |  |  | X |
| #8 |  |  | X |
| #9 |  |  | X |
| #10 | X |  |  |
| #11 | X |  |  |
| #12 | X |  |  |
| #13 | X |  |  |
| #14 | X |  |  |
| #15 |  |  | X |
| #16 |  |  | X |
| #17 |  |  | X |

## 6.1	Solution #1: Support Pre-Mission Planning and In-Mission Monitoring Flight

### 6.1.1	Key Issue mapping

This solution addresses KI#1 aspects.

### 6.1.2	Description

Enhance NEF services to support service exposure and interactions between MNOs and UTM functions for supporting pre-mission flight planning and in-mission flight monitoring for UAVs, is one of the study aspects in KI#1. This solution proposes to enhance NEF services, enable USS/UTM to request NEF assistance for pre-mission flight planning and in-mission flight monitoring.

### 6.1.3	Procedures and Parameters

#### 6.1.3.1	Procedure for NEF Assist Pre-mission Flight Planning

Figure 6.1.3.1-1: Procedure for NEF Assist Pre-mission Flight Planning

1.	The UAV (or UAV-C) establishes a PDU Session for communication with the USS/UTM as described in clause 5.2.3 of TS 23.256 [2].

2.	The UAV (or via its paired UAV-C) requests Pre-mission flight planning service from USS/UTM. The request message includes identifier of the UAV (e.g. GPSI, CAA-Level UAV ID), information of the starting and ending points for the flight, requirements on the flight route (e.g. on time), and may include candidate flight route(s) if available.

3.	The USS/UTM derives information for the Pre-mission flight planning request and decides to request assistance information from NEF.

NOTE 1:	The content of Pre-mission flight planning service information derived at USS is out of scope.

NOTE 2:	The solution uses request/response since considers such approach is more suitable for this scenario. Pre-mission flight planning, after receiving the planned route from USS, the UAV is expected to plan by following the route. There is no need to keeps receiving the notifications. When it is required to keep receiving notification during flight, the solution belongs to in-mission monitoring.

4.	The USS/UTM sends Pre-mission flight planning assist request to NEF. The request message includes identifier of the UAV (e.g. GPSI), information of the starting and ending points for the flight, requirements on the flight route (e.g. on time), candidate flight route(s) (received from the UAV or local derived at the USS/UTM), and accuracy level of predictions relevant to the flight planning.

5.	The NEF maps parameters included in the request from the USS/UTM to information used by the 3GPP system (e.g. map the geographical area into an area of interest that is represented by a list of Cell IDs, gNB IDs or TAIs). The NEF determines services needed for the request and relevant NFs, e.g. NEF service on UAV tracking and mode (UAV location reporting mode, UAV presence monitoring mode, List of Aerial UEs in a geographic area), NWDAF analytics service (Movement Behaviour analytics), GMLC service (e.g. Ranging/Sidelink Positioning location).

6.	If the NEF cannot satisfy the requirements on flight planning (e.g. cannot get predictions relevant to the flight planning from NWDAF with the required accuracy level), the NEF responses to the USS/UTM to reject the Pre-mission flight planning assist request, and may include the detail reason.

7.	If being rejected in step 6, the USS/UTM may response to the UAV (or its paired UAV-C) to reject the Pre-mission flight planning request, may include the detail reason if available.

8.	The NEF performs steps 2-5 in clause 5.3.4 of TS 23.256 [2], and filter list of UAVs in the areas of interest.

9.	The NEF subscribes/requests for notification on Movement Behaviour analytics provided by NWDAF as defined in clause 6.21.4 of TS 23.288 [6]. The subscribe/request message include identifier of the UAV (e.g. GPSI) obtained in step 4. The other parameters included in the request are described in clause 6.1.3.3. The list of UAVs in the areas of interest output from step 8 can be used as inputs for the NEF to request the NWDAF on Movement Behaviour analytics.

10.	The NEF may request GMLC service(s), e.g. Ranging/Sidelink positioning location as defined in clause 6.20.3 of TS 23.273 [7]. The list of UAVs in the areas of interest output from step 8 can be used as inputs for the NEF to request the GLMC service on Ranging/Sidelink Positioning location.

11.	The NEF generates assistance information for pre-mission flight planning based on the information and analytics from steps 8-10. The assistance information may be the best matched route among the ones provided from the USS/UTM in step 4, or potential flight route(s) if candidate flight route(s) is not provided in step 4.

12.	The NEF responses to the USS/UTM with the assistance information.

13.	The USS/UTM decides flight planning by using the assistance information and flight planning mechanism which is out of scope.

14.	The USS/UTM sends response to the UAV (or its paired UAV-C) with the planned flight route.

NOTE 3:	USS/UTM are used to represent the consumer of the enhanced NEF services in Figure 6.1.3.1-1, the consumer can also be other entities, e.g. TPAE.

#### 6.1.3.2	Procedure for NEF Assist In-mission Flight Monitoring

Figure 6.1.3.2-1: Procedure for NEF Assist In-mission Flight Monitoring

NOTE 1:	The NFs in Figure 6.1.3.2-1 represents all the other NFs beside GMLC and NEF/UAS NF involved in the procedure.

1.	The USS/UTM sends In-mission flight monitoring assist request to NEF. The request message includes identifier of the UAV (e.g. GPSI) and information of the flight route, monitoring mode (e.g. UAV location monitoring, fight route monitoring, flight environment monitoring, Ranging/Sidelink positioning location monitoring), report requirements (e.g. report format (event triggered or periodically), assistance information (e.g. the monitoring results, information generated from the monitoring results)), and may include planned fight route if fight route monitoring, list of UAVs in the areas of interest if flight environment monitoring, preset value for distance if Ranging/Sidelink positioning location monitoring.

2.	NEF maps the parameters in the request from the USS/UTM to information used by the 3GPP system. The NEF determines services needed for the request and relevant NFs, e.g. NEF service on UAV tracking (UAV location reporting mode, UAV presence monitoring mode, List of Aerial UEs in a geographic area), GMLC service (Ranging/Sidelink Positioning location).

3.	If the NEF cannot provide the requested assistance information for In-mission flight monitoring, the NEF responses to the USS/UTM to reject the In-mission flight monitoring assistance request, may include the detail reason if available.

4.	For UAV location monitoring, the NEF executes steps 2-5 in clause 5.3.2 of TS 23.256 [2] to obtain the location of the target UAV.

5.	For fight route monitoring, the NEF executes steps 2-4 in clause 5.3.3 of TS 23.256 [2]. Upon to the request in step 1 for reporting, the NEF may compare the monitored flight route with the planned fight route (if provided in step 1) and generate assistance information on whether the UAV is flight in right route.

6.	For flight environment monitoring, the NEF executes steps 2-5 in clause 5.3.4 of TS 23.256 [2] to obtain the information of aerial UEs in the same geographic area of the target UAV.

7.	For Ranging/Sidelink positioning location monitoring, the NEF requests GLMC service for Ranging/Sidelink Positioning location results as described in clause 6.20.4 of TS 23.273 [7], the procedure of SL-MT-LR for periodic, triggered Location Events (steps 1-20 for initiation the monitoring, steps 21-31 for monitoring periodic). The list of UAVs in the areas of interest output from step 6 can be used as inputs to step 7 for NEF to request GLMC service for Ranging/Sidelink Positioning location, if the UAVs is unknown in the areas of interest.

8.	The NEF notifies the USS/UTM the monitoring results (e.g. the location of the target UAV, indication on whether the UAV is flight in right route, the target UAV presence in areas of interest, the information of aerial UEs in the same geographic area of the target UAV, the distance of other UAVs and the target UAV and whether the distance is smaller than a preset value).

Steps 4-8 may be repeated for report monitoring results periodically.

NOTE 2:	USS/UTM are used to represent the consumer of the enhanced NEF services in Figure 6.1.3.2-1, the consumer can also be other entities, e.g. TPAE.

#### 6.1.3.3	Parameters in Request for Movement Behaviour Analytics

The USS acting as an Application Function communicates with the NEF which corresponds to the NF consumer in clause 6.21.4 of TS 23.288 [6].

Editor's note:	The final list of parameters is FFS.

The USS can either subscribe to notifications from the NEF (i.e. a Subscribe-Notify model) or request a single notification from the NEF (i.e. a Request-Response model). The USS request contains the following parameters:

-	Analytics ID = "movement behaviour";

-	Target of Analytics Reporting: any UE;

-	Analytics Filter Information:

-	Area of Interest (AOI): restricts the scope of the movement behaviour analytics to the provided area. The AOI may be described as shown in clause 5.5 of TS 23.273 [7];

-	Optionally, the list of analytics subsets that are requested among those specified in clause 6.21.3 of TS 23.288 [6];

-	An Analytics target period indicates the time period over which the statistics or predictions are requested;

-	Optionally, preferred level of accuracy of the analytics;

-	Optionally, preferred level of accuracy per analytics subset (see clause 6.21.3 of TS 23.288 [6]);

-	Optionally, preferred granularity of location information: "longitude and latitude level";

-	Optionally, preferred orientation of location information: ("horizontal", "vertical", "both"); and

-	Optionally, maximum number of objects.

### 6.1.4	Impacts on services, entities and interfaces

UAS NF/NEF:

-	Enhance for supporting pre-mission flight planning assistance service.

-	Enhance for supporting in-mission flight monitoring assistance service.

NWDAF:

-	Enhance the inputs and the outputs of Movement Behaviour Analytics.

USS/AF:

-	Enhance for supporting UAV/UAV-C trigger pre-mission flight planning service.

-	Enhance for supporting in-mission flight monitoring service.

Editor's note:	It is FFS other possible impacts related to services, entities and interfaces.

## 6.2	Solution #2: UAV flight path deviation exposure

### 6.2.1	Key Issue mapping

This solution aims to resolve

Key Issue #1, "Enhancement of NEF services to support service exposure and interactions between MNOs and UTM functions".

### 6.2.2	Description

The solution also makes use of UE flight path reporting feature (AAM). It is assumed, in this solution, the UE flight path reporting is also implemented/available in 5G NR. Alternatively, or complementary, this solution can use MDT reports that also bring an insight of the radio conditions and UE location. The solution further proposes enhancements where the reporting can be triggered by an NF/AF (e.g. NEF) via the AMF on the NG RAN and then the NG RAN configures the UE to report its flight path (see steps 2 and 4a in the call flow diagram).

### 6.2.3	Procedures

#### 6.2.3.1	Monitoring & reporting of UAV flight path deviation

The USS/UTM (acting as an external AF) can access 3GPP exposed services using public identifier of the UE (used in the UAV). Below pre-conditions apply:

-	When UAV authentication/authorization procedure is used:

-	UAV is authenticated during PDU session establishment (or optionally at 5GS registration) by USS/UTM.

-	3GPP system provides the public identifiers (e.g. 3GPP UAV ID, IP address) of the UE (used in the UAV) to the USS/UTM.

-	USS/UTM uses the public identifier of the UE to access 3GPP exposed services.

-	Or UAV authentication/authorization procedure is not used:

-	UAS operator provides the serving MNO information and the public identifier (e.g. 3GPP UAV ID) of the UE (used in the UAV) to the USS/UTM.

-	USS/UTM uses the public identifier of the UE to access 3GPP exposed services.

Editor's note:	It is FFS, how the UAS operator knows about GPSI/3GPP UAV ID without the authorization procedure?

Figure 6.2.3.1-1: Monitoring and reporting of UAV flight path deviation using NWDAF analytics.

1.	The USS/UTM sends a request for flight path monitoring of a UE (used in a UAV) to NEF/UAS NF. The request contains at least the following information elements:

a.	UE ID (i.e. 3GPP UAV ID) or group of UE Ids)

b.	the authorized flight path in 3D way points along with estimated time,

c.	the acceptable deviation from the authorized flight path expressed in 3D distance deviation and/or time deviation (absolute or relative units).

d.	Conditions, e.g. that may alter or define differently the acceptable 3D deviation including the dimension of time.

2.	NEF subscribes to new flight path monitoring analytics from NWDAF. The request contains, at least the following information elements:

a.	(a new) Analytics ID=FlightPathDeviation,

b.	the UE ID,

c.	the authorized flight path in 3D way points along with estimated time,

d.	the acceptable deviation or the means (e.g. function) to derive the acceptable deviation considering certain conditions (e.g. weather) from the authorized 3D flight path in the form of:

i	Acceptable 3D deviation in distance (absolute or relative units).

ii.	Acceptable deviation in time (absolute or relative units).

Editor's note:	It is FFS how to set Analytics target period as this new Analytics ID is neither for statistics nor for predictions.

3.	Implementation alternatives:

a.	Or NWDAF subscribes to periodic UE location reporting via GMLC or MDT reports.

b.	NWDAF collects UE flight path report from NG RAN (via OAM) and the UE location reporting via GMLC.

4.	NWDAF analyses/compares the UE location report received through GMLC or MDT reports and the UE flight path report collected from NG RAN with the authorized flight path information received in the request from USS/UTM (forwarded by NEF):

a.	NWDAF first verifies if the UE reported flight path information matches with the corresponding location reports received from GMLC (requested in 4b). If the two reports do not match, then the NWDAF reports it to the USS/UTM (via NEF) in step 6.

b.	If the UE reported flight path information matches with the GMLC reported location, the NWDAF then further compares the UE reported flight path with the authorized flight path information received in the request from USS/UTM. The NWDAF also takes into consideration "acceptable deviation" information received as input in the request from USS/UTM.

If a flight path deviation is found, the NWDAF reports it to USS/UTM via the NEF. The report also contains information on detected deviation points (e.g. 3D location) or detected deviation time or both, and confidence of the reported deviation.

NOTE 1:	The NWDAF can collect location information from 5GC and/or RAN without using NEF and create analytics for acceptable deviation path.

NOTE 2:	The NWDAF is intended to be used as a monitoring entity for path deviation without needing any historical data analysis. The NWDAF can use existing data collection from RAN/GMLC.

Editor's note:	No new MDT enhancement is assumed for this solution. It is FFS, whether existing MDT reports are applicable and/or accurate for UAV UEs.

### 6.2.4	Impacts on services, entities and interfaces

NWDAF:

-	Support for new analytics ID and analytics event.

NEF:

-	Potential impact to existing exposure API due to new analytics, if any.

## 6.3	Solution #3: UAV flight planning and monitoring

### 6.3.1	Key Issue mapping

This solution addresses Key Issues #1 which proposes to enhance the notification on QoS Sustainability Analytics mechanism to the AF (e.g. USS/UTM) for pre-mission flight planning and in-mission flight monitoring.

The enhancement of QoS Sustainability Analytics supported by NWDAF is to consider the height information, corresponding to the UAV flight path when deriving the likelihood of a QoS change for an Analytics target period in the future in a certain area.

### 6.3.2	Description

An AF (e.g. USS/UTM) may request notifications on QoS Sustainability Analytics from the NEF for an indicated geographic area or a path of interest and a target time interval in order to adjust the application behaviour in advance according to the potential QoS change. How the AF (e.g. UTM/USS) makes use of QoS Sustainability Analytics is outside of 3GPP scope.

AF can in principle reuse what is defined for V2X Application Server in clause 5.4.5.2 of TS 23.287 [8]. The main new aspect is to introduce the "height" information in the location information. The location or path information of UAVs differs from that of ground vehicles considering the UAV flight path information. As described in TS 38.300 [9], the Aerial UE flight path information consists of a number of waypoints defined as 3D locations and may contain a time stamp per waypoint. Based on this information, the solution adds "height" information as an attribute to the location information.

The added "height" information is an important input for the NWDAF in order to generate a realistic QoS Sustainability analytics for UAV pre-mission flight planning and in-mission flight monitoring. While the antennas of the base station have a specific azimuth, the antennas are often directed to the ground, and the high-altitude coverage is thus rather limited (called "ground cells" in the following description). It can however be expected that there are also dedicated base stations deployed to provide a certain coverage for the high altitude (called "sky cells" in the following description).

Editor's note:	The assumption that such dedicated "sky cells" exist need to be confirmed during this study.

If the UAV flight path is on a low altitude or close to the ground, the performance measurement information of the ground cells should be used for the generation of the analytics. If the UAV flight path is at higher altitudes, the coverage area of the ground cell is rather small (or there may be even no coverage anymore). Hence, the QoS Sustainability analytics for the UAV at a certain height should be only based on the performance measurement information of the sky cells. If the altitude is in a range where both types of cells (i.e. ground and sky cells) could provide connectivity, the NWDAF could combine the performance measurement information of both types of cells (e.g. applying certain altitude dependent weights).

Editor's note:	It is FFS whether the NWDAF can be enabled to calculate the coverage area of "ground cells" for different heights.

The NWDAF should therefore take the UAV height information into consideration in order to generate the QoS Sustainability analytics based on the performance measurement information of the most appropriate cells from OAM. In consequence, the AF has more accurate information available for UAV pre-mission flight planning and in-mission flight monitoring.

Editor's note:	It is FFS whether the NWDAF can be configured with information about the available "sky cells" and their coverage area.

### 6.3.3	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

### 6.3.4	Impacts on services, entities and interfaces

AF (USS/UTM):

-	Provide location information (waypoints defined as 3D locations) to NEF when request QoS Sustainability Analytics.

NWDAF:

-	Consider 3D location for deriving QoS Sustainability Analytics in order to identify the performance measurement information of the most appropriate ground and/or sky cell and to estimate the coverage area of the ground cell at the respective height (Input and Output data of the Analytics is not impacted).

## 6.4	Solution #4: Network-supported Tactical Deconfliction

### 6.4.1	Key Issue mapping

This solution applies to Key Issue #2.

### 6.4.2	Description

#### 6.4.2.1	Introduction

Sense and Avoid (SAA), Detect and Avoid (DAA), or tactical deconfliction systems are technologies that allow uncrewed aerial vehicles (UAVs) and drones to integrate safely into civilian airspace, avoiding collisions with other aircraft, buildings, power lines, birds and other obstacles. These systems observe the environment surrounding the drone, decide whether a collision is imminent, and generate a new flight path to avoid collision.

Traditional UAV sense and avoid systems may combine data from communication interfaces (e.g. A2X), a number of sensors, using sensor fusion algorithms, image recognition and artificial intelligence to provide the best outcome. Data is fed back to the drone on-board computer and/or drone flight controller, which can then decide on the best evasive maneuver or flight path correction to avoid collision. A reliable onboard DAA system is crucial for obtaining a waiver for flight operations in many jurisdictions that typically otherwise require human observers or ground-based observation systems along the entire flight path. DAA systems are thus key to unlocking commercially viable BVLOS (beyond visual line of sight) drone operations that provide services such as inspection and cargo delivery over extremely long distances.

Regulations regarding tactical deconflictions are being developed, and they traditionally refer to solution components that employ a ground component in order to:

-	Not having to rely fully on RPS (remote Pilot Station)/UAVC (UAV controller)/GCS (Ground Control Station)/human pilot;

-	Leverage the high level of automation that is already widely available in aerial vehicles, while at the same time not relying solely on aerial vehicle awareness of surrounding traffic;

-	Leverage ground networks ability to have higher spatial awareness of traffic.

In the scope of this solution, we use the term Network-based DAA (NWDAA) to refer to tactical deconfliction solutions that utilize a network-based or ground-based component to collect, elaborate, and distribute tactical deconfliction information.

#### 6.4.2.2	Solution Overview

The solution proposed assumes the deployment of an AIML-based localized USS/UTM function tailored specifically to provide NWDAA services to UAVs and UTM. We identify this function as Localized DAA Server (LDS) for tactical deconfliction. We assume that LDS is a functionality separate from USS/UTM due to the different service it provides, and the fact that LDS may be provided by the MNO.

The solution is based on the following assumptions:

-	The LDS provides a subscription-based service, available to all aerial UEs or a subset of the aerial UE (identified by the aerial subscription introduced in Rel. 15).

-	LDS nodes elaborate spatial awareness based on information collected on UAVs and potentially other aerial vehicles.

-	LDS collects spatial awareness information from a set of sources: UAVs, sensors (e.g. radar station, ADS-B receivers, A2X receivers, etc.). The location of such sensors and interfacing with the LDS is dependent on deployment.

-	LDS may belong to the MNO domain or may be provided by a 3rd party.

Editor's note:	How trust of LDS is assured (including LDS authorization) and how a 3rd party LDS can access NWDAF and NEF services is FFS.

-	A UAV may be visible to multiple LDSs. Only one LDS interacts and serves a UAV UE, but multiple LDSs may consider a specific UAV UE presence and information.

-	LDS could interact with NDWAF and leverage information obtained by NDWAF.

NOTE 1:	No new services for NWDAF are proposed in this solution, but if new services related to UAV become available, the LDS is assumed to be able to access those.

-	Each LDS serves a specific area

-	an LDS implements conflict detection and traffic separation algorithms and collision notification features across one or more mobile network cells.

-	the LDS service area is configured in the LDS and may be configured in the PLMN. It is assumed that an LDS may communicate the serving area to UTM.

-	The LDS obtains situational awareness information about UAVs (e.g. UAV identity, location, flight vectors) and potentially additional situational awareness about the airspace (e.g. obstacle identification, crewed aircraft that transmit ADS-B).

-	The LDS may leverage NEF/UAS NF services for interaction with UTM/USSs

-	LDS may obtain specific UAVs flight plans.

-	LDS may report potential or actual conflicts to UTM, and receive instructions and configuration information from the UTM.

-	LDS may provide via NEF exposure Aerial congestion information API and UAV information to USS to support USS in flight authorization.

-	LDS is assumed to communicate with USS via UAS NF/NEF, and as such LDS is not required to know the serving USS, and the serving USS is not required to know the serving LDS function, i.e. no information about the serving USS is provided to the LDS, and the LDS is not required to discover the serving USS. The UAV communicates with LDS providing the current CAA-Level UAV ID, and the LDS uses the CAA-Level UAV ID to discover the serving UAS NF and provide the information together with the CAA-level UAV ID. The UAS NF can then forward such information to the USS(s) serving the UAV associated to the CAA-Level UAV ID received from the LDS.

-	for any asynchronous request from USS to the LDS, it is assumed that the LDS serving a UAV ID registers itself with the serving UAS NF for the UAV ID using the CAA-Level UAV ID to indicate which UAV UE it is serving.

-	The serving LDS for a UAV may change during the flight path of the UAV, and the UAV context is moved between UAVs using application layer mechanisms.

-	Connectivity between the UAV and the LDS is over user plane, and it is assumed that a dedicated PDU session is established to enable such communication (e.g. to leverage edge connectivity).

Editor's note:	Re-use of existing PDU sessions (e.g. for UAV-UTM communications) is FFS.

NOTE 2:	Edge relocation mechanisms may be used to relocate the serving LDS for a UAV.

#### 6.4.2.3	Solution Architecture

The solution assumes that the LDS is deployed by the MNO at the edge of the network, and communications over the UAV and the LDS are carried out over user plane.

The LDS can interface with NEF/UAS NF using existing or new/extended service exposure functionality.

### 6.4.3	Procedures

#### 6.4.3.1	Overall Information Flow

The following logical flow is assumed:

Figure 6.4.3.1-1: Overall Information Flow for NWDAA Solution

NOTE:	Steps 4 to 7 are exchanged at application layer and are outside the scope of 3GPP. They are described here to provide an overview of how the solution would work as a whole.

1.	The UAV registers to the 5G System using existing procedures, with the enhancements described in this solution.

2.	The UAV establishes connectivity with the LDS server. This may include authorization and authentication of the UAV to be enabled to access LDS services. This may also include the UAV receiving policy information on LDS availability, and on LDS communication (what information is to be exchanged, and at what frequency, etc.). The LDS server registers itself with the UAS NF of the UAV and provides the CAA-Level UAV ID of the UAV.

3.	At any time the LDS may retrieve UAV-related information from USS via NEF, e.g. the UAV flight plan or any other information that supports LDS functionality.

Editor's note:	Whether an existing NEF service is used or a new one needs to be defined will be determined during normative phase.

4.	The UAV sends information to the LDS according to the policies received in step 2. This includes information about the UAV (e.g. CAA-Level UAV ID, location, flight vector, etc.), and information received by the UAV via A2X and regarding other UAVs visible to the UAV. The LDS may also receive information from other sources, e.g. ADS-B receivers, radar stations, etc.

5.	Upon detecting a potential conflict, the UAV may indicate the conflict to the LDS and request deconfliction assistance.

6.	Based on information received at step 4 or based on conflict detection performed autonomously by the LDS, the LDS may send deconfliction information or warn the UAV of potential conflict. The UAV may exchange such information with the UAVC for deconfliction actions.

7.	Based on information received at step 4 or based on an urgent and serious conflict detection performed autonomously by the LDS, the LDS may send emergency directives to one or more UAVs. It is expected that emergency directives are processed locally and autonomously by the UAV, and the UAV may inform the UAVC.

8.	At any time the LDS may interact with the USS via NEF to provides aerial congestion/conflict information to support flight planning in USS.

9.	At any time the USS may subscribe to LDS service notifications regarding a specific UAV.

It is assumed that a UAV UE that is capable of LDS indicates it supports NWDAA in NAS signaling capabilities as described in the following sections.

LDS service may not be available ubiquitously in the whole serving network; therefore, an indication of LDS Service Area needs to be provided to the UAV. The network may provide to the UAV UE that has indicated LDS support and is subscribed to LDS whether LDS is available or not and may provide an LDS Service Area (e.g. whole PLMN, current RA, list of TAs, etc.). This may be generated by the AMF based on OAM configuration and provided to the UE directly in MM signaling or passed to the SMF and returned to the UAV UE upon successful establishment of PDU session for UAS services.

#### 6.4.3.2	UAV-LDS Connectivity Establishment for LDS@Edge

UAV UE that is capable of LDS provides the LDS indication during a PDU Session Establishment for UAS services as described in TS 23.256 [2] and may be for a PDU session established for C2, for C2 and UTM services, or dedicated for LDS services depending on UAV UE configuration as to which DNN shall be used for LDS services. UUAA-SM may be extended to contain an indication that the UAV is requesting LDS service, to enable UTM to authorize the use of LDS service and to authorize the specific LDS server. The UUAA-SM may return to the SMF information about the LDS server(s).

Upon successful PDU session establishment for LDS services, the UAV is provided by the SMF with information on how to reach the LDS, and the UAV establishes connectivity with the LDS directly using application layer signaling outside the scope of this solution.

NOTE:	It is expected that security mechanisms may be required to ensure the UAV is authorized to access the LDS. This may require the LDS to perform an additional UUAA procedure, or an additional security mechanism may be required.

Figure 6.4.3.2-1: UAV-LDS Connectivity Establishment for LDS@Edge

1.	The UAV UE registers to the network.

2.	UAVE UE provide the LDS indication to the SMF in the PDU Session Establishment request. This may be during the establishment of a PDU session for UAS services as described in TS 23.256 [2], for a PDU session established for C2, for C2 and UTM services, or a dedicated PDU session for LDS services depending on UAV UE configuration as to which S-NSSAI/DNN shall be used for LDS services.

3.	The SMF verifies the LDS subscription and may perform UUAA-SM, depending on the type of PDU session being used.

4.	Upon successful PDU session establishment, the SMF provides indication that LDS is available and authorized, and provides information on how to reach the LDS.

5.	The UAV UE establishes connectivity with the LDS directly using application layer signaling outside the scope of this solution.

### 6.4.4	Impacts on services, entities and interfaces

The following entities are impacted:

-	UE:

-	NWDAA capability indication in 5GSM;

-	ability to receive policies for LDS information transmission;

-	ability to receive LDS@Edge addressing information transmission.

-	SMF:

-	verify LDS service subscription;

-	configuration of LDS service availability;

-	provide UAV UE with LDS information;

-	receive LDS information and policies from PCF (optional).

-	NEF/UAS NF:

-	extend existing services or define new services for interaction between LDS and UTM.

-	UDM:

-	optional indication of NWDAA service subscription.

The following impacts on services are identified:

-	UUAA-SM extended to carry LDS information (information may be in existing payload or new payload and does not impact NEF/UAF NF);

-	Existing NEF services may be extended, or new ones defined, to enable the UTM to subscribe to LDS services (e.g. even reporting for potential or identified traffic conflicts);

-	Existing NEF services may be extended, or new ones defined, to enable the UTM to provide UAV flight plan and additional UAV information to LDS.

NOTE:	The specific impacts on NEF services will be identified during normative phase.

## 6.5	Solution #5: Support Network-assisted DAA with Existing 5GC Services

### 6.5.1	Key Issue mapping

This solution addresses KI#2 aspects.

### 6.5.2	Description

Study which existing information collected and generated in the 5GS can be utilised to enable NWDAA is one of the study aspects in KI#2. This solution proposes to utilize the GMLC service on Ranging/Sidelink Positioning location and Relative Proximity predictions on collision generated at NWDAF to support Network-assisted DAA.

USS as AF/LCS client may send the relative positioning request to GMLC, including UAV UE1 ID (UE1 GPSI) and UAV UE2 ID (UE2 GPSI), time slot for relative positioning and including a distance threshold. GMLC determines a scheduled location, time, and frequency to get the location information of UE1 and UE2 at the same time within the given time slot.

The calculation of relative distance of the pair of UAV1 and UAV2, it may be performed GMLC or USS.

(1) For GMLC-based option:

GMLC initiates MT-LR for UAV and UAV-C simultaneously, by including scheduled location time included in the request sent to AMF, as described in clause 6.1.2 of TS 23.273 [7]. GMLC calculates the relative positioning result based on the UAV UE1 location and UAV UE2 location from step 2 and step 3. USS as AF/LCS client sends the relative positioning request to GMLC, including UAV UE1 ID (UE1 GPSI) and UAV UE2 ID (UE2 GPSI), time slot for relative positioning and including a distance threshold. GMLC determines a scheduled location, time, and frequency to get the location information of UE1 and UE2 at the same time within the given time slot. GMLC checks the relative position result is lower than the distance threshold. GMLC reports the relative position result to USS within the given time slot.

(2) For USS-based option:

USS requests the UAV1 and UAV2 location to the GMLC as described in clause 6.1.2 of TS 23.273 [7], and a scheduled location time is used.

NOTE:	USS can request same or different GMLC for the UAV1 and UAV 2 location, e.g., depending on which PLMN the UAV registers.

After receiving the location of UAV1 and UAV2, USS calculates the relative positioning result for UAV UE1 location and UAV UE2 location from step 1 and step 2. USS may send the DAA notification to UAV or the corresponding UAV controller via application layer, so that the UAV can change the flight path timely to avoid the collisions. The specific action of USS is out of scope of this specification.

USS may be replaced by TPAE in this option.

### 6.5.3	Procedures and Parameters

#### 6.5.3.1	Procedure for UAV/UAV-C Triggered Network-assisted DAA

Figure 6.5.3.1-1: Procedure for UAV/UAV-C Triggered Network-assisted DAA

1.	The UAV (or UAV-C) establishes a PDU Session for communication with the USS as described in clause 5.2.3 of TS 23.256 [2].

2.	The DAA service may be triggered by UAV(s) or UAV-C. The UAV(s) (via its paired UAV-C) or the UAV-C requests DAA service from USS. The request message includes identifier of the UAV(s) (e.g. GPSI(s), CAA-Level UAV ID(s)). USS derives information on DAA service and decides to subscribe/request to 5GC for GMLC service on Ranging/Sidelink Positioning location and/or Relative Proximity predictions on collision from NWDAF.

NOTE 1:	The other content of DAA service information derived at USS is out of scope.

3.	The USS may request GLMC service via NEF for Ranging/Sidelink Positioning location results, i.e. the relative positioning between UAV and UAV-C as described in clause 6.20.3 of TS 23.273 [7] for one notification, or clause 6.20.4 of TS 23.273 [7] for notifications (steps 1-20 for initiation monitoring, steps 21-31 for monitoring periodic).

USS as AF/LCS client may send the relative positioning request to GMLC, including UAV UE1 ID (UE1 GPSI) and UAV UE2 ID (UE2 GPSI), time slot for relative positioning and including a distance threshold and an indication whether the calculation of relative distance of the pair of UAV1 and UAV2 is performed by GMLC or USS. GMLC determines a scheduled location time, and frequency to get the location information of UE1 and UE2 at the same time within the given time slot.

(1) For GMLC-based option:

GMLC initiates MT-LR for UAV and UAV-C simultaneously, by including scheduled location time included in the request sent to AMF, as described in clause 6.1.2 of TS 23.273 [7]. GMLC calculates the relative positioning result based on the UAV UE1 location and UAV UE2 location from step 2 and step 3. USS as AF/LCS client sends the relative positioning request to GMLC, including UAV UE1 ID (UE1 GPSI) and UAV UE2 ID (UE2 GPSI), time slot for relative positioning and including a distance threshold. GMLC determines a scheduled location, time, and frequency to get the location information of UE1 and UE2 at the same time within the given time slot. GMLC checks the relative position result is lower than the distance threshold. GMLC reports the relative position result to USS within the given time slot.

(2) For USS-based option, GMLC replies to USS of the location of UAV 1 and UAV 2.

NOTE 2:	USS may request same or different GMLC for the UAV1 and UAV 2 location, e.g. depending on which PLMN the UAV registers.

4.	The USS may subscribe or request notification on Relative Proximity predictions provided by NWDAF via NEF by invoking Nnef_AnalyticsExposure_Subscribe service operation as defined in clause 6.1.1.2 of TS 23.288 [6] or Nnef_AnalyticsExposure_Fetch service operation as defined in clause 6.1.2.2 of TS 23.288 [6]. The subscribe/request message include identifier of the UAV(s) (e.g. GPSI(s)). The other parameters included in the request are described in clause 6.5.3.3.

5.	After NEF receive the request from the USS, the NEF interacts with the NWDAF as described in the procedure in clause 6.19.4 of TS 23.288 [6]. The NEF maps the parameters in the request from the USS to information used by the 3GPP system.

6.	If the NEF receives the response from the NWDAF, the NEF notifies the USS with the Relative Proximity predictions by invoking Nnef_AnalyticsExposure_Notify service operation for a Subscribe-Notify model as defined in clause 6.1.1.2 of TS 23.288 [6] or Nnef_AnalyticsExposure_Fetch service operation for a Request-Response model as defined in clause 6.1.2.2 of TS 23.288 [6].

7.	If receiving the location of UAV1 and UAV2, USS calculates the relative positioning result for UAV UE1 location and UAV UE2 location from step 1 and step 2. The USS estimates the potential collision based on the information received in step 3 and/or the predictions from step 6. The USS informs the UAV-C(s) the potential collision. The message may include collision alert, predicted time of collision, CAA-level UAV IDs of the paired UAVs which may collision (e.g. UAV 1 and UAV 2), deconflicting specific parameters (e.g. trajectory correction information to avoid collision).

USS may be replaced by TPAE.

8.	The UAV-C(s) informs its paired UAV(s) the potential collision, and information received from the USS, include collision alert, predicted time of collision, CAA-level UAV IDs of the paired UAVs which may collision, and deconflicting specific parameters (e.g. trajectory correction information to avoid collision).

9.	UAVs performs operations to avoid collision.

9a.	If both the two UAVs which may collision (e.g. UAV 1 and UAV 2) been informed by their paired UAV-C(s), the UAVs can be steered to avoid collision in accordance with the received information (e.g. trajectory correction information to avoid collision) and using mechanisms that are out of scope for 3GPP.

9b.	If only one of the two UAVs which may collision (e.g. UAV 1) been informed by its paired UAV-C, the UAV 1 triggers conflict resolution procedure with UAV 2 as described in steps 4-7 in clause 5.6.1 of TS 23.256 [2].

#### 6.5.3.2	Procedure for AAM Triggered Network-assisted DAA

Figure 6.5.3.2-1: Procedure for AAM Triggered Network-assisted DAA

1.	As described in clause 5.7.2 steps 1-6 of TS 23.256 [2], the UAV(s) listens for signals on the correspondingly destination Layer-2 ID configured for the used service type. The AAM scans the airspace over the area/arena for UAV(s), retrieves for each detected UAV the corresponding Remote-ID, and establishes a PC5 direct communication link with the discovered UAV. Using the PC5 unicast direct communication link the AAM and the UAV establishes a bidirectional communication channel for exchange of messages.

2.	The AAM establishes a PDU Session for communication with the USS as described in clause 5.2.3 of TS 23.256 [2].

3.	The AAM may request network assist DAA service from USS. The request message includes identifier of the UAV(s) (e.g. GPSI(s), CAA-Level UAV ID(s)). USS derives information on DAA service and decides, e.g. to request GMLC service for Ranging/Sidelink Positioning location, to subscribe/request to NWDAF for Relative Proximity predictions on collision.

NOTE:	The other content of DAA service information derived at USS is out of scope.

4.	The USS requests GMLC service for Ranging/Sidelink Positioning location, and/or subscribes/requests notification on Relative Proximity predictions provided by NWDAF via NEF, as described in clause 6.5.3.1 steps 3-6.

5.	The USS estimates the potential collision based on the received information/analytics in step 4. The USS informs the AAM the potential collision. The message may include collision alert, predicted time of collision, CAA-level UAV IDs of the paired UAVs which may collision (e.g. UAV 1 and UAV 2), deconflicting specific parameters (e.g. trajectory correction information to avoid collision).

6.	The AAM provides the determined steering policy to the specific UAVs (e.g. UAV 1 and UAV 2) according to the information received in step 5, and the UAVs are steered to avoid collisions in accordance with received policy and using mechanisms that are out of scope for 3GPP, as described in clause 5.7.2 steps 7-8 of TS 23.256 [2].

#### 6.5.3.3	Parameters in Request for Relative Proximity Analytics

The USS acting as an Application Function communicates with the NEF which corresponds to the NF consumer in clause 6.19.4 of TS 23.288 [6].

The USS can either subscribe to notifications from the NEF (i.e. a Subscribe-Notify model) or request a single notification from the NEF (i.e. a Request-Response model). The USS request contains the following parameters:

-	Analytics ID = "Relative Proximity";

-	Target of Analytics Reporting: a UE, a group of UEs;

-	Analytics Filter Information:

-	Area of Interest;

-	An individual or set of direction(s) of interest;

-	Number of UAVs to be accounted for relative proximity (i.e. the number of UAVs for which one UAV may report proximity information);

-	One or several attributes to be accounted for relative proximity (i.e. additional information that can be provided in addition to distance between two UAVs): velocity, average speed, orientation, mobility trajectory;

-	Preferred level of accuracy of the analytics;

-	Maximum number of objects;

-	An Analytics target period indicating the time period over which the predictions are requested.

### 6.5.4	Impacts on services, entities and interfaces

UAS NF/NEF:

-	Enhance Nnef_AnalyticsExposure service, e.g. additional parameters.

NWDAF:

-	Enhance the inputs and the outputs of Relative Proximity Analytics.

Editor's note:	It is FFS if existing data is enough or enhancement is needed.

USS/AF:

-	Enhance for supporting UAV/UAV-C trigger DAA service.

-	Enhance for supporting AAM trigger network assist DAA service.

-	Support of calculate the relative positioning between any two UAV UEs.

GMLC:

-	Support of calculate the relative positioning between any two UAV UEs.

Editor's note:	It is FFS other possible impacts related to services, entities and interfaces.

## 6.6	Solution #6: UTM requests information used for DAA

### 6.6.1	Key Issue mapping

This solution address key issue #2: Network-assisted/ground-based mechanism for DAA (Detect And Avoid) with 5GS information.

### 6.6.2	Description

In this solution, it is the UTM request the following existing information from 5GC used for DAA:

-	Sidelink positioning location results when certain trigger events occur. The trigger events are described in clause 6.20.4 of TS 23.273 [7], e.g. one trigger event occurs if the distances between at least one UE of the n UEs and each of the other UEs are such that any distance for the one UE is less than the threshold.

-	List of Aerial UEs in a geographical area as specified in clause 5.3.4 of TS 23.256 [2],

-	NWDAF analytics specified in TS 23.288 [6], e.g. UE mobility analytics, relative proximity analytics.

### 6.6.3	Procedures

Figure 6.6.3-1: UTM requests information from 5GC

1-3.	USS/UTM obtains the aerial UE list in a geographical area by invoking the existing procedure of obtaining list of Aerial UEs in a geographic area in clause 5.3.4 of TS 23.256 [2].

4-5.	USS/UTM requests the relative proximity analytics of the UEs obtained in steps 1-3. The steps 4-5 represent the existing procedure for NWDAF providing relative proximity analytics in clause 6.19.4 of TS 23.288 [6].

6-8.	USS/UTM triggers the existing procedure for SL-MT-LR for periodic, triggered location events in clause 6.20.4 of TS 23.273 [7]. The threshold type could be set to threshold type a, in this case, the trigger event occurs if the distances between at least one UE of the n UEs obtained in steps 1-3 and each of the other UEs are such that any distance for the one UE is less than the threshold.

Based on the NWDAF analytics, event report and flight path of the aerial UEs, if a collision is detected, the USS/UTM initiates a collision avoidance/conflict resolution procedure which is out of 3GPP scope.

### 6.6.4	Impacts on services, entities and interfaces

UTM/USS:

-	Obtain aerial UE list in a geographical area.

-	Obtain the NWDAF analytics on relative proximity analytics of the aerial UEs.

-	Obtain the sidelink positioning location results when certain trigger events occur. The trigger events are described in clause 6.20.4 of TS 23.273 [7], e.g. one trigger event occurs if the distances between at least one UE of the n UEs and each of the other UEs are such that any distance for the one UE is less than the threshold.

## 6.7	Solution #7: Mobility Enhancements for enforcements of NTZ

### 6.7.1	Key Issue mapping

This solution applies to Key Issue #3.

### 6.7.2	Description

#### 6.7.2.1	Introduction

The solution addresses the following aspects:

-	How to ensure an aerial UE respects no-transmit zones. This includes the following scenarios:

-	mobile network cells overlapping completely or partially with the NTZ and using the restricted frequency bands of the NTZ.

-	support NTZ of any size, depending on regional requirements.

-	the provisioning of NTZ-related information to the UAV UE.

-	how to allow the enforcement for both aerial UEs in connected mode and aerial UEs in idle mode.

#### 6.7.2.2	Solution Overview

The solution assumes the following:

-	NTZ information is provided to the PLMN functions (depending on the solution options described below) via OAM by external parties, with the idea that at national level there will be entities in charge of defining NTZs and providing related information to PLMNs.

Editor's note:	Whether NEF services are extended to enable providing dynamic NTZ information to the 5GC is FFS.

-	NTZ information refer to a geospatial description of an NTZ (in two dimensions or three dimensions to consider height restrictions) and the restricted frequencies corresponding to such area.

Editor's note:	The solution is based on a set of assumptions related to the expected behavior of a UAV UE when in the NTZ (i.e. the NTZ area and the restricted frequency bands). Depending on verification of such assumptions, the solution may need to be revised.

The solution provides multiple options which cater for different size of NTZs with respect to the size of the cells impacted by NTZs:

-	Extension of Service Restrictions to cater for NTZs: this solution is suitable for large NTZs that impact a variety of cells in a specific area.

-	use of NTZ policy configuration in the UE.

-	Per-cell indication of the presence of NTZs: this solution assumes that mechanisms are adopted in RAN to provide information to UAV UEs with respect to the overlap of an NTZ with the cell (i.e. the whole cell is not suitable for transmission for a UAV UE) in order to ensure that the UAV UE does not transmit at all in the NTZ area of a cell that overlaps partially or fully with an NTZ. The solution is mostly suitable for scenarios where the NTZ covers the majority or entirety of a cell.

A PLMN may use one or more of these solutions depending on local policies and the size of the NTZs impacting the PLMN.

In the solutions below, the following behavior is assumed:

-	The UAV UE never transmits any signalling or data in the area of a cell corresponding to an NTZ (i.e. the geospatial area corresponding to the NTZ and the frequency band(s) corresponding to the NTZ).

-	the UAV UE is provided by the network with NTZ-related information as described in the solutions below.

-	the UAV UE deactivates the AS layer or ceases to transmit any data when entering an area corresponding to the NTZ.

NOTE:	It is assumed that if the UE can receive DL data, the UE cannot perform any actions that require any transmission while in the area corresponding to an NTZ.

Editor's note:	Whether the UE is allowed to receive DL data in an area corresponding to and NTZ is FFS.

-	the UAV UE may perform a registration update before entering the NTZ with an indication that the UAV UE is about to enter an NTZ, so that the network considers the UAV UE unreachable but registered, similarly to the mechanisms adopted in previous releases for high latency communication behavior. The UAV UE does so based on the NTZ information it receives as described below. The UAV UE would then perform re-registration when exiting the NTZ. This enables the network to know the exact status of the UAV UE.

-	for connected mode UAV UEs, it is expected that the network is configured to know which cells correspond to an NTZ and the RAN will attempt to avoid handing over a UAV UE to a cell corresponding to an NTZ. However, in some scenarios the only suitable cell for a handover may be a cell impacted by an NTZ (location and frequency bands), and the UE behavior described below must apply. Optimizations in the case of connected mode handover to minimize loss of connectivity and ensure the respect of NTZs need to be discussed further with RAN WGs.

-	for IDLE mode mobility, the UAV UE behavior is described below.

Editor's note:	Additional/what parameters need to be considered to properly describe NTZ and actual restrictions/enforcement aspects and roles of NTZ in the core and radio network are FFS.

Editor's note:	Coordination with RAN WGs are required to progress the solution to ensure that RAN (eNB/gNB) has inputs required to enforce UE compliance with NTZ.

Editor's note:	How the solution can address change of NTZ information is FFS.

#### 6.7.2.3	Extension of Service Restrictions for NTZs

In this solution component, a UAV UE supporting NTZ restrictions provides an indication in 5GMM of support of NTZ Restrictions.

The CN receives the NTZ information (e.g. via OAM from external party) and maps the NTZ information to a Restricted Transmission Area (RTA). If the AMF determines that the supporting UE is an UAV UE, i.e. the UE provided indication of NTZ support, the UE has an aerial subscription, and after successful UUAA procedure when it is performed, then the AMF configures the UE with this Restricted Transmission Area. If a UAV UE does not provide the indication, the AMF may deregister the UE and may provide a cause code (and a backoff timer) to indicate not to re-register for a time for this PLMN.

The Restricted Transmission Areas is similar to a Non-Allowed Area, but it is explicitly identified as being different from a Non-Allowed Area to induce a different behavior in the UAV UE. The concept is that the UE is not allowed transmit at all the RTA in the frequencies corresponding to the NTZ.

Based on the RTA information, the UAV UE behavior differs from a traditional UE which, when entering a non-allowed area, may still act as a regular UE in limited state and transmit for emergency services, and is banned from performing any PLMN reselection. An UAV UE entering an RTA shall not initiate any service request or any signaling, including emergency services. When entering an RTA, the UAV UE shall behave as a UE with no suitable serving cell banned from performing any emergency services. An UAV UE in an RTA shall not respond to any paging from the network. When entering an RTA, the UAV UE is allowed to use this event to trigger PLMN reselection.

In order to provide NTZ information as part of the RTA:

-	the AMF may be configured with NTZ information and provide RTA to the UAV UE based on the NTZ information after the AMF determines it is a supporting UAV UE (i.e. the UE provided indication of NTZ support), that the UE is an aerial UE based on aerial subscription, and after successful UUAA procedure when it is performed.

-	Alternatively, the AMF may be configured to know that there is at least one NTZ in the area served by the AMF but may not create any RTA information. During the registration, the PCF which is configured with NTZ information, may create an RTA for the UAV UE and return it to the AMF which in turn provides it to the UAV UE.

The UDM and the PCF may update the Service Area Restrictions of a UE at any time. For UE in CM-IDLE state in an RTA marked as NTZ, the AMF shall store the updated service area restriction and update the UE upon next signalling interaction with the UE. For a UE located in an RTA marked as NTZ, the AMF shall not initiate paging for a UE to update Service Area Restrictions with Generic UE Configuration Update procedure.

#### 6.7.2.4	NTZ Restriction Policies

This component of the solution re-uses concepts similar to the PC5 policy configuration that was adopted in previous releases to define geofence where certain frequencies are not allowed. However, in the case of NTZs, a blocklist of frequencies associated to the specific area are provided, whereas in the case of PC5 a allowlist was provided. In the solution, the UE is configured with NTZ Restriction Policies that identify the area corresponding to the NTZ and the banned frequency or frequencies. The UE shall not transmit data or signaling when in the area of the NTZ and is being served by the frequencies identified in the NTZ Restriction Policies. The description of the NTZ may be in the form of a 2D or 3D (to include specific altitudes) geo-area/polygon.

In this solution, a UAV UE supporting NTZ restrictions provides an indication in 5GMM of support of NTZ Restrictions.

The UE may be provided with NTZ Restriction Policies in one of the following alternatives:

-	Option 1: From AMF. During the registration procedure, if the AMF determines that the supporting UE is an UAV UE, i.e. the UE provided indication of NTZ support, the UE has an aerial subscription, and after successful UUAA procedure when it is performed, then the AMF provides the NTZ Restriction Policies applicable to the UAV UE and the current registration area. This option requires all AMF serving areas impacted by NTZ to be configured with NTZ information.

-	Option 2: From PCF. When the UAV UE is in a TA where there is at least NTZ, or when the registration Area that the AMF assigns to the UAV UE contains at least one NTZ, and if the AMF determines that the supporting UE is an UAV UE, i.e. the UE provided indication of NTZ support, the UE has an aerial subscription, and after successful UUAA procedure when it is performed, then the AMF provides a new indication that an NTZ is present in the registration area. The UAV UE triggers the retrieval of NTZ Restriction Policies from the PCF using existing procedures. Alternatively, the AMF may indicate to the PCF during the registration procedure that NTZ are present in the registration area and the PCF triggers NTZ Restriction Policies delivery to the UAV UE. This solution reduces the impact on AMFs by requiring only AMFs that serve area impacted by NTZs to be configured to know there is an NTZ, without the need to impact other AMFs and without requiring AMFs to be aware of the complete NTZ information.

-	Option 3: from NTZ AF. This option assumes that a third-party service is present outside MNOs to provide NTZ-specific information to UAV UEs. This may be a service provided by regulators or other third parties. As in option B, When the UAV UE is in a TA where there is at least NTZ, or when the registration Area that the AMF assigns to the UAV UE contains at least one NTZ, and if the AMF determines that the supporting UE is an UAV UE, i.e. the UE provided indication of NTZ support, the UE has an aerial subscription, and after successful UUAA procedure when it is performed, then the AMF provides a new indication that an NTZ is present in the registration area, and may provide assistance information (e.g. URL) to instruct the UAV UE how to reach the AF. Upon receiving the indication and the assistance information, the UAV UE uses existing a suitable user plane connection or establishes an appropriate user plane connection (e.g. the UAV UE may be configured with a specific DNN/S-NSSAI for such connectivity) to the AF and uses application layer signalling to retrieve the information (out of scope of the solution).

If in a PLMN multiple options are supported, this solution assumes that what the UAV UE receives in option 3 takes priority over the information received with other options.

#### 6.7.2.5	Per-cell indication of the presence of NTZs:

For potential solutions where a per-cell indication is provided to UAV UEs on the presence of NTZs (independently of the mechanism adopted to do so by e.g. RAN solutions), three categories of UEs need to be considered:

1.	new UEs (i.e. Rel. 19) that support NTZ restriction functionality: in a cell where the NTZ indication applies, the UAV UE shall consider the cell barred.

2.	pre Rel. 19 non-UAV UEs (i.e. terrestrial UEs): these UE should be allowed to use the cell either by ignoring the indication or not receiving it.

3.	pre-Rel.19 UAV UEs that do not support NTZ restriction: solutions must enable such UAV UEs to consider the cell barred or not suitable.

We assume that the network would need to be aware of which of the three categories a UE belongs to, and we assume that new Rel.19 UAV UEs indicates to the network the ability to support NTZ restrictions. CN and RAN are already aware if this is an aerial UE based on the UE subscription being an aerial subscription, which since Rel. 15 is indicated to the RAN when the UE context is provided to the RAN.

Solutions that provide a per-cell indication of the presence of an NTZ shall be able to support all three categories of UAVs with the expected UE behavior described above. Specifically, if RAN WGs develop such solution, the solution must allow non-UAV UEs to use the cell without restriction and must block all UAV UEs from using this cell. This includes also UAV UEs of release 18, not just UAV UEs of Release 19. Moreover, such solutions would need to consider that RAN needs to be configured with NTZ information in order to provide any per-cell indication.

NOTE:	Coordination with RAN WG is required to consider such solutions.

#### 6.7.2.6	Solution Architecture

No architectural modifications are assumed by the proposed solution.

### 6.7.3	Procedures

#### 6.7.3.0	General

NOTE:	It is assumed that the following procedures take place when the UAV UE is outside an area corresponding to an NTZ.

#### 6.7.3.1	Extension of Service Restrictions for NTZs

Figure 6.7.3.1-1: Delivery of NTZ information via Service Area Restrictions

1.	The UE registers indicating support for NTZ.

2.	If the AMF determines that the supporting UE is an UAV UE, i.e. the UE provided indication of NTZ support, the UE has an aerial subscription, and after successful UUAA procedure when it is performed, then the AMF decides to configure the UE with this Restricted Transmission Area. If a UAV UE does not provide the indication, the AMF may deregister the UE and may provide a cause code (and a backoff timer) to indicate not to re-register for a time for this PLMN. The AMF may be configured with NTZ information and provide RTA to the UAV UE based on the NTZ information.

3.	Alternatively, the AMF may be configured to know that there is at least one NTZ in the area served by the AMF but may not create any RTA information. During the registration, the PCF which is configured with NTZ information, may create an RTA for the UAV UE and return it to the AMF which in turn provides it to the UAV UE.

4.	The AMF confirms the registration and returns the RTA information to the UAV UE. Based on the RTA information, a UAV UE entering an RTA shall not initiate any service request or any signaling, including emergency services. When entering an RTA, the UAV UE shall behave as a UE with no suitable serving cell banned from performing any emergency services. An UAV UE in an RTA shall not respond to any paging from the network. When entering an RTA, the UAV UE is allowed to use this event to trigger PLMN reselection.

#### 6.7.3.2	NTZ Restriction Policies

Figure 6.7.3.2-1: Delivery of NTZ restriction policies to UAV UEs

1.	The UE registers indicating support for NTZ.

2.	If the AMF determines that the supporting UE is an UAV UE, i.e. the UE provided indication of NTZ support, the UE has an aerial subscription, and after successful UUAA procedure when it is performed, then the AMF determines that NTZ Restriction Policies applicable to the UAV UE and the current registration area need to be provided to the UAV UE.

3a.	In option 1, the AMF may retrieve the NTZ Restriction Policies from the PCF.

3b. in option 1, the AMF returns the NTZ Restriction Policies to the UAV UE.

4a.	In option 2, the AMF provides a new indication that an NTZ is present in the registration area.

4b.	In option 2, based on the indication in 4a the UAV UE triggers the retrieval of NTZ Restriction Policies from the PCF using existing procedures. Alternatively, the AMF may indicate to the PCF during the registration procedure that NTZ are present in the registration area and the PCF triggers NTZ Restriction Policies delivery to the UAV UE.

5a.	In option 3, the AMF provides a new indication that an NTZ is present in the registration area, and may provide assistance information (e.g. URL) to instruct the UAV UE how to reach the AF.

5b.	In option 3, upon receiving the indication and the assistance information in step 5a, the UAV UE uses existing a suitable user plane connection or establishes an appropriate user plane connection (e.g. the UAV UE may be configured with a specific DNN/S-NSSAI for such connectivity) to the AF.

5c.	In option 3, the UAV UE uses application layer signalling to retrieve the information from the AF.

### 6.7.4	Impacts on services, entities and interfaces

The following entities are impacted:

-	UE:

-	indication of NTZ support for UAV UEs;

-	receive and process NTZ information, and modify idle mode mobility and connected mode mobility based on received NTZ information as described in options above	;

-	abstain from transmission in areas corresponding to NTZ;

-	for per-cell solutions, the UE can receive per-cell indication of NTZ.

-	AMF:

-	configuration by OAM to be aware of presence of NTZ in serving area;

-	potential configuration of NTZ information or assistance information to support NTZ information retrieval;

-	restrain from paging a UAV UE that is in an NTZ.

-	PCF:

-	may be configured with NTZ information (area and frequencies) depending on solution options.

## 6.8	Solution #8: Network support for NTZ management

### 6.8.1	Key Issue mapping

This solution addresses part of KI#3 regarding non-NTZ-support UE.

### 6.8.2	Description

This solution addresses Key Issue #3 "Support of No Transmit Zones".

Considering the scenario that the UE doesn't support functions defined for NTZs wants to perform flight mission via 5G network in NTZ. There is a need to align NTZ setting with 5G network, so 5G network can manage aerial UEs based on the right aerial information (NTZ). In order to avoid the impact on non-aerial UEs, AMF needs to determine whether the UE is an aerial UE, and if yes, AMF can compare the NTZ information with the UE's local information to determine whether the user can initiate registration via 5G network.

Editor's note:	Applicability of whether an UAV can access the network while moving into or in a NTZ needs to be confirmed and any solution associated with such assumption will be further evaluated once clarification of this issue has been performed.

### 6.8.3	Procedures

Figure 6.8.3-1: Procedure for network support for NTZ

0.	The AMF can store the NTZ information which is configured locally or obtained from the UTM/USS via UAS-NF/NEF.

1.	The UE, which doesn't support functions defined for NTZs, sends a Registration request message and, and it shall provide a CAA-level UAV ID of the UAV if configured with one.

2.	AMF could determine whether the UE is an aerial UE according to the CAA-level UAV ID or aerial subscription, and if yes, AMF can compare the NTZ information with the TA where the UE is to determine whether the UE can continue registration.

3.	AMF sends Registration Accept/Registration Reject message to UE.

Editor's note:	The indication sent by the AMF to prevent subsequent registration or mobility registration attempt is for FFS.

4.	The AMF can update and store the new NTZ information configured locally or obtained from the UTM/USS via UAS-NF/NEF.

5.	The UE sends a mobility registration message due to the UE mobility.

6.	AMF could determine whether the UE is an aerial UE according to the CAA-level UAV ID or aerial subscription, and if yes, AMF can compare the NTZ information with the TA where the UE will move to, to determine whether the UE can continue mobility registration.

7.	AMF sends mobility registration Success/mobility registration Failure message to UE.

### 6.8.4	Impacts on Existing Nodes and Functionality

AMF:

-	Obtain data from UTM/USS referring to NTZ and store it as NTZ information.

## 6.9	Solution #9: Enabling NTZ support for aerial UEs

### 6.9.1	Key Issue mapping

This solution addresses KI#3 aspects.

### 6.9.2	Description

The solution assumes that an aerial UE indicates its capability to respect no-transmit zones (NTZ), and UE's subscription data has a record about that, as Operators have to enforce the NTZ respect, and the subscription-based approach is one possibility, as described below.

Several options/alternatives are considered when comes to obtaining NTZ information - preconfiguration at the relevant network nodes (e.g. gNodeB (gNB)/eNodeB(eNB), AMF/MME) as well as a network-based configuration of NTZ parameters (e.g. geographical area in form of coordinates (i.e. latitude and longitude), restricted frequency band(s), altitude/elevation etc.).

Editor's note:	Additional/what parameters need to be considered to properly describe NTZ and actual restrictions/enforcement aspects and roles of NTZ in the core and radio network are FFS.

In order to perform network-based configuration of NTZ parameters, it is assumed that there needs to be operator's AF with trust relation for this purpose. This AF can be part of UTM (Uncrewed Aerial System Traffic Management) , which is outside operator's trust domain, and therefore, the AF will invoke a service operation towards an UAS NF / NEF so that the NTZ information can be transferred towards RAN nodes (i.e. gNBs in case of 5GC and eNB in case of EPC), core network (CN) entities and to the affected aerial UEs.

Editor's note:	Whether UEs in NTZs are allowed to use UL for emergency support is FFS and depends on LS reply.

Current solution assumes that the aerial UEs supporting NTZ will not require emergency services support.

Until otherwise indicated, it is assumed that the NTZ does not allow UEs to send uplink data but allows reception of downlink data from the network. Also, Aerial UEs within NTZ(s) are able to communicate with the network for mobility/registration procedures for the purposes of keeping connectivity to the network.

The main principles/steps for the considered solution options/alternatives.

1.	Provisioning RAN (gNB/eNB) with a set of NTZ information (e.g. geographical area in form of coordinates (i.e. latitude and longitude), restricted frequency band(s), altitude/elevation etc.), see Figure 6.9.3-1:

1(a).	RAN is provided with NTZ information via a node-level signalling from operator's AF via UAS NF/NEF, PCF and AMF using the AM Policy Association Establishment/Modification procedures (as described in clauses 4.16.1 and 4.16.2 of TS 23.502 [4]).

1(b).	RAN node(s) are configured by OAM with the existing NTZ information that can be requested by the AF/UTM from network's nodes in the area they serve (e.g. based on RAN node location, Tracking Area(s) identified by a list of tracking area identities (TAI) or a list of cell identities)

1(c):	AMF/MME is pre-configured (e.g. local configuration) with NTZ information; and AMF/MME provides the NTZ information to relevant gNB/eNB nodes via non-UE associated signalling (e.g. during NG interface Setup/Reconfiguration procedure), UE-associated signalling (e.g. during Initial Context Setup/Modification procedure or PDU session resource management procedures).

To provide RAN nodes with new/updated NTZ information from the AF/UTM (i.e. triggered by UTM), a node-level signalling from operator's AF/UTM can be used for Option 1(a), 1(b) and 1(c), whereas reprovisioning via OAM can be done only for Option 1(b).

2.	UE's subscription data include an additional record at UDM, indicating that the UE/UAV is compliant to NTZs requirements. This allows the operator and the network to enforce NTZ respect and to deliver the NTZ information only to UEs with the corresponding subscription. This indication is made available/delivered to RAN nodes (gNB/eNB) so they can enforce the NTZ respect. Additionally, UE's subscription data at UDM can indicate which set(s) of NTZ are allowed to be disobeyed by high-priority UEs (e.g. first responders).

3.	Providing the NTZ information to a UE with the corresponding subscription:

3(a).	RAN (gNB/eNB) sends the NTZ information to a UE using the RAN defined procedure.

3(b).	The NTZ information is sent to a UE from a core network node (e.g. UAS NF/NEF via an AMF) or from the serving AMF using Non-Access Stratum (NAS) signalling during UE's registration in a Registration Accept message.

The UE/UAV stores the received NTZ information until the new NTZ information is provided or deleted, e.g. by explicit signalling from the core network (for instance, via AMF NAS signalling) due to request from the AF/UTM.

Editor's note:	It is FFS if SMF/UPF/PGW-U need to be impacted due to enforcement of no UL data transmission and Aerial UE movement in and out of NTZ(s).

Editor's note:	Coordination with RAN WGs are required to progress the solution to ensure that RAN (eNB/gNB) has inputs required to enforce UE compliance with NTZ.

### 6.9.3	Procedures

The main procedural steps to provision RAN nodes (gNB/eNB) with the NTZ information is shown in Figure 6.9.3-1.

Editor's note:	The current procedures assume 5GC signalling. The adaptation for EPC is FFS.

Figure 6.9.3-1: Procedure to provision RAN nodes (gNB/eNB) with the NTZ information

Option 1: node-level signalling.

1.	An AF (UTM) sends to the UAS NF/NEF an Naf_Authentication_Notification request to provide new/updated information about NTZ for the UE/UAV. The AF/UTM includes GPSI, CAA-Level UAV ID, PDU Session IP address if available and the NTZ information in the re-authentication/authentication data update request.

Editor's note:	Whether to use the existing service operation (e.g. Naf_Authentication_Notification) or to design a new for this specific purpose is FFS.

NOTE 1:	The similar request can be used by the AF/UTM to send an update about the NTZ information when e.g. the AMF/MME is preconfigured with the set of NTZ information or when the previously delivered information is not valid anymore.

2.	The UAS NF/NEF translates the AF-provided NTZ information (e.g. geographical area in form of coordinates (i.e. latitude and longitude), restricted frequency band(s), altitude/elevation etc) into 3GPP identifiers, e.g. a list of Tracking Area Identifiers (TAIs) or a list of cell IDs, RFSP Index.

3A or 3B.	The UAS NF/NEF discovers a PCF handling AM Policy for the UE (Option 3A) or a serving AMF (Option 3B); for that the NEF invokes the NRF discovery service and uses UE's identity.

3C.	The UAS NF/NEF uses UDM service (Nudm_UECM_Get operation as specified in clause 5.2.3.2.4 of TS 23.502 [4]) to get an NF ID of the AMF serving the UE, i.e. with UE Context. In congestion with that, the UAS NF may invoke another service operation to UDM to retrieve UE's subscription data and check whether the UE is compliant to NTZs requirements allowing the operator and the network to enforce NTZs.

If the UE's subscription does not have the NTZ indication, the UAS NF notifies the AF/UTM by sending the response message and include the information about the results inside this message.

NOTE 2:	It is up to UTM to decide for which purpose this information can be used; it is outside 3GPP scope.

4A.	The UAS NF/NEF sends to the discovered PCF an Npcf_AMPolicyAuthorization_Create/Update request containing the translated NTZ information, e.g. a list of TAs/Cell IDs, and the original AF-provided NTZ information (prior the translation by the NEF).

5A.	After receiving the NTZ information, the PCF takes policy decision and then may initiate the AM Policy Association Modification procedure for the UE/UAV as described in clause 4.16.2.2 of TS 23.502 [4] to provide the AMF with the NTZ information.

The PCF does not initiate the AM Policy Associations Modification and rejects (i.e. responds with the failure cause) the Npcf_AMPolicyAuthorization_Create/Update request in cases when the PCF does not receive information about the UE's capability to respect NTZ during the AM Policy Association Establishment (see step 16 in clause 4.2.2.2.2 of TS 23.502 [4]). The PCF responds to the Npcf_AMPolicyAuthorization_Create/Update request (not shown in Figure 6.9.3-1), and the UAS NF/NEF responds to the AF (using Naf_Authentication_Notification response, not shown in Figure 6.X.3-1) to inform the UTM that the UE/UAV does not have capability required.

4B and 4C.	If the UE's subscription data contains the indication that the UE is compliant with NTZ requirements, the UAS NF/NEF sends an Nnef_Authentication_Notification request to the AMF/MME, containing the translated NTZ information, e.g. a list of TAs/Cell IDs, the original AF-provided NTZ information to the target AMF. If the UE subscription does not indicate the NTZ compliance, the UAS NF/NEF responds to the AF/UTM to notify that the UE/UAV does not have capability required.

6A and 6B.	If the UE subscription check (i.e. compliance to NTZ requirements) has not been performed earlier in the procedure, the AMF/MME checks, based on the subscription data retrieved during the registration, whether the UE is compliant with NTZ requirements.

7.	Based on the NTZ information, the AMF determine the relevant RAN node IDs and then sends N2 messages containing the NTZ information and include an indication to enforce (whenever required NTZ respect) to all applicable RAN nodes.

Editor's note:	What actions and how RAN uses the delivered NTZ information is up to RAN WGs to study, and it is FFS.

NOTE 3:	RAN can use the NTZ information to adjust information broadcasting, e.g. prevent UEs from moving from RRC_IDLE/INACTIVE states to RRC_CONNECTED state.

Option 2: OAM configuration.

1.	RAN node(s) are configured by OAM with the existing NTZ information that can be requested by the AF/UTM from network's nodes in the area they serve (e.g. based on RAN node location, Tracking Area(s) identified by a list of tracking area identities (TAI) or a list of cell identities).

2.	AMF sends an N2 message with indication that the RAN needs to enforce NTZ after the Registration Completion.

Option 3: AMF preconfiguration with NTZ information.

0.	AMF is preconfigured with sets of NTZ information that can be requested by the AF/UTM.

1.	To request a specific NTZ requirements, the AF/UTM may use e.g. an index (a number) to a specific set; Steps (some or all), 3B/C, 4B/C, 6A, and 7 as in Option 1 are executed.

2.	Delivery of the NTZ information together with the indication to enforce NTZ during NG interface Setup/Reconfiguration procedure, during Initial Context Setup/Modification procedure or PDU session resource management procedures.

The main procedural steps to provide UEs with the NTZ information is shown in Figure 6.9.3-2.

Figure 6.9.3-2: Procedure to providing UEs with the NTZ information

Option 1: RAN Mechanism

1.	UE preforms registration procedure as described in clause 4.2.2.2.2 of TS 23.502 [4].

2.	AMF retrieves the subscription data from the UDM and checks whether the UE is compliant with NTZ requirements. If so, the AMF/MME sends N2 message with the indication to RAN that it shall enforce the NTZ whenever required.

3.	RAN sends the received NTZ information, how this is done is FFS and need coordination with RAN WGs.

Option 2: Registration Accept

1.	UE to RAN: UEs sends a Registration Request message and include parameters as specified in clause 4.2.2.2.2 of TS 23.502 [4]. Additionally, if the UE has capabilities to respect no-transmit zones, the UE includes the indication about this capability to the network.

2.	RAN to AMF: Once RAN selects an AMF, the RAN sends an N2 message containing N2 parameters (specified in step 3 of clause 4.2.2.2.2 of TS 23.502 [4]) and the received Registration Request message from the RAN (as described in step 1). The AMF stores information UE's support for NTZ consideration in the UE Context.

3.	If the AMF sees in the registration request information about UEs capability to respect NTZ, the AMF check whether UE subscription data includes indication about UEs compliance with NTZ requirements. If the UE is compliant with NTZs, the AMF may verify the UE's location before replying with a Registration Accept/Reject message.

4.	If the UE is allowed to operate at its present location, the AMF sends to the UE a Registration Accept message and includes inside the message the previously received NTZ information.

If the UE is not allowed to operate at its present location, the AMF may either: (1) send a Registration Reject message with a cause value indicating that the UE is not allowed to operate at the present UE location, or alternatively, (2) send a Registration Accept message in which the AMF includes a UE Radio Capability ID and/or RFSP Index the UE is allowed to operate.

5.	If the UE receives a Registration Reject message with cause value indicating the UE/UAV is not allowed to operate in the present location due to NTZ requirements, the UE may attempt to perform an Emergency Registration.

The overall NTZ procedure is as shown in Figure 6.9.3-3, and it includes:

Figure 6.9.3-3: Procedure to support UE compliance with the provided NTZ information

1.	NTZ information and the indication to enforce NTZ delivery to the relevant RAN nodes as described in Figure 6.9.3-1.

2.	NTZ information delivery to UEs with the corresponding subscription, as described in Figure 6.9.3-2.

3.	UE location determination. As an option, and depending on UTM requirements, the AMF may use the UE mobility event notifications to get information about UE's presence in Area(s) of Interest, as specified in clause 5.3.4.4 of TS 23.501 [5].

4.	In case of UE's presence in the AoI: once the AMF detects the UE's presence in the NTZ, the AMF sends an N2 message with a new value of 'Index to RAT/Frequency Selection Priority' (RFSP Index) to RAN nodes. Based on the received RFSP Index, the NG-RAN nodes decide about redirecting UEs to different frequency layers or RATs (as specified in clause 5.3.4.3.1 of TS 23.501 [5]).

Editor's note:	How to detect UE transmit in NTZ and how to handle corresponding UL traffic is FFS.

5.	If there is any change in the provisioned NTZ information, the RAN nodes and UEs need to be re-provisioned with the newest NTZ information.

6.	For UEs/UAV in CM_CONNECT with RRC_INACTIVE state or in CM_IDLE, the AMF may initiate, based on the local policy, the Network Triggered Service Request procedure as described in clause 4.2.3.3 of TS 23.502 [4] before executing steps 3-5. If the UE is not updated while it was in IDLE, the AMF updates the UE next time it becomes available gain (e.g. RRC_CONNECTED).

### 6.9.4	Impacts on services, entities and interfaces

Impact depends on which options/alternatives selected for delivering NTZ information to UEs and RAN nodes and for updating them about any change from the AF/UTM. Overall, the impacts could be the following:

AF:

-	Invoking a Naf_Authentication_Notification or a new service to request NTZ compliance from the network,

-	Receiving notifications when UEs enter or disobey NTZ requirements.

UAS NF/NEF:

-	Discovering a PCF handling AM policy,

-	Discovering AMF serving the UE,

-	Retrieving information about UE's subscription and checking whether the UE can comply with NTZ,

-	Translating the NTZ information to 3GPP identifiers such as TA, NG-RAN node identifiers, cell IDs.

PCF:

-	Invoking AM Policy Association Modification due to NTZ.

AMF:

-	Determining UE's location/presence in the area of interest,

-	Checking UE's subscription for NTZ compliance,

-	Informing PCF about UE's capability to respect NTZ,

-	Informing RAN nodes about enforcing NTZ requirements,

Editor's note:	It is FFS whether and how (e.g. based on which information) the indication to enforce NTZ is provided by AMF to RAN.

-	Informing PCF about UE's capability to respect NTZ,

-	Sending notifications when UE enters the NTZ.

RAN:

-	delivering NTZ information to the relevant UEs,

-	Enforcing NTZ compliance.

UE/UAV:

-	respecting NTZ requirements,

-	announces its capability to comply with NTZ requirements.

UPF:

-	Handling (e.g. discard, etc.) UL data traffic for UEs in NTZ (FFS if/when needed).

UDM:

-	Inclusion of NTZ enforcement indication and provide to AMF.

Editor's note:	It is FFS if any additional parameter(s) is needed or existing subscription is sufficient.

## 6.10	Solution #10: Enabling 5GC support for multiple USS serving different geographical areas

### 6.10.1	Key Issue mapping

This solution addresses KI#1 aspect related to 5GC support for multiple UAS Service Suppliers (USS-s) serving different geographical areas that fully or partly fall within UAV’s flight path.

### 6.10.2	Description

The focus in this scenario is with two USS-s – USS 1 serving a geographical area confined within tracking area (TA 1), TA 2 and TA 3 and USS 2 serving a geographical area confined within TA 4 and TA 5; each of the TAs has different number of cells etc., see Fig. 6.10.2-1.

1

2

3

4

5

6

7

8

1

9

2

3

4

5

6

1

2

3

1

2

3

4

5

6

1

2

3

4

5

6

7

USS 2

USS 1

TA 5, USS 2

TA 2, USS 1

TA 3, USS 1

TA 4, USS 2

TA 1, USS 1

Border between USS 1 and USS 2

Border cells: TA1_8, TA1_9, TA2_6, TA3_2, TA3_3, TA4_1, TA4_2, TA4_3, TA4_6

UE location 1: TA3_1

Figure 6.10.2-1: USS-s serving different geographical areas confined within TAs.

Each USS is responsible for its geographical area that does not identically match tracking areas of a 3GPP network, however, it can be assumed that TAs and Cells IDs can be used to describe/map these geographical areas. Whenever required, a network exposure function (NEF) can be used to make a translation from a geographical description of an area to 3GPP identifiers such as TAs and cell IDs. In the considered scenario, there are border cells between areas served by USS 1 and USS 2, respectively. These cells are: TA1_9, TA1_8, TA3_2, TA3_3, TA2_6, TA4_1, TA4_2, TA4_3 and TA4_6. Whenever a UAV moves inside any of the border cells, a changeover from one USS to another is triggered. For that an AMF needs to notify the associated NEF/UAS NF about the UAV presence in a border cell/TA so that the NEF/UAS NF could notify a serving USS about the event (i.e., that a UAV moves inside a border cell/TA). After receiving the notification from the NEF/UAS NF, the serving USS can continue communication with a target USS to ensure a changeover of the UAV to the intended USS.

Assumption: A UAS Service Supplier (USS) is aware of neighbouring USS-s, border areas that it has with its neighbour USS, possible UAV border-crossing points and can communicate with each other using means outside the 3GPP scope.

After receiving the notification about UAV’s presence in a border cell/TA, the serving USS can perform a number of actions depending on USS capabilities, UAV flight path (e.g. a targeted destination, a specific border-crossing point, arrival time) and other information relevant to the flight path.

-	If a UAV uses a USS for pre-mission flight planning and indicates a destination in a border cell/TA under responsibility of another USS (e.g. TA4_2 as shown in Fig. 6.10.2-1), there is no need to perform the UAV changeover if it is assumed that both or several USS-s can serve cells/TAs within a border region. However, if the UAV requested destination is farther than a border region (e.g. TA5_7) the changeover from a serving USS (USS 1) to a target USS (USS 2) is required. The situation is similar when there is a strict separation between geographical areas served by each USS. For instance, when USS 1 cannot serve in TA4_1, TA4_2, TA4_3 and TA4_6. It is assumed that when such multi-USS deployment is supported, the USS-s have pre-established knowledge (e.g. via pre-configuration, OAM, other proprietary means etc. that are not in scope of the study but assumed) of the neighbouring USS-s in order to perform changeover.

-	If a UAV uses a USS for pre-mission flight planning and indicates a destination “deep” inside a geographical area served by another USS (e.g., TA5_7, see Fig. 6.10.2-2), there could be different ways how a serving USS could handle such case once it determines that UAV’s target destinations is outside its responsibility/service area.

-	Scenario#1: USS 1 plans flight path(s) only until a possible border-crossing point(s) in a border cell(s)/TA(s) (TA3_2 in this case) and triggers a changeover/communicates with USS 2 only when the UAV enters TA3_2. Before doing so, after receiving the notification about the UAV’s presence in a border cell/TA, additionally USS 1 may also trigger the service to determine whether UAV’s direction (which is evaluated based on notifications for UAV’s deviation from the assigned/expected trajectory, movement behaviour etc.) is towards outside of its service area in which case USS 1 starts a changeover process with a target USS.

-	Scenario#2: USS 1 plans path(s) until a possible border-crossing point(a) in a border cell(a)/TA(s) with USS 2 (similarly as in Option 1), however, when/if the UAV enters any cell/TA inside USS 2 service area, the UAV is required to make another request for the remaining route to its destination in TA5_7 or event to reconnect if the UAV moves to another PLMN (i.e., USS 2 serves another PLMN). In any case, the UAV needs to communicate with a target USS (i.e., USS 2) in order to plan its path(s) onwards to its target destination in TA5_7, and afterwards the target USS (USS 2) requests a serving NEF/UAS NF for assistance with pre-mission flight planning for the remaining path to the destination point.

-	Scenario#3: USS 1 as a serving USS triggers communication with a targeted USS (USS 2) to jointly determine flight path(s) to a targeted destination in TA5_7. USS 1 with a help of a serving NEF/UAS NF makes flight path(s) (one or more) between the starting point and the selected border-crossing point(s), whereas USS 2 decides on suitable flight path(s) (one or more) across its area from the selected border-crossing point(s) to the destination point. USS 2 provides this information to USS 1, and USS 1 provides this information to the UAV. Furthermore, to ensure a smooth changeover from USS 1 to USS 2, USS 1 and USS 2 can request (via NEFs/UAS NFs) the respective AMFs (i.e., an AMF serving TA3_2 and an AMF serving TA4_2 assuming the planned route goes through them) to get notified when the UAV leaves/enters the respective border cells/TAs. During the changeover a serving USS provides indications to a target USS about exposure services of the UAV. The target USS, after completion of the changeover from USS 1, may invoke the relevant exposure services via a serving NEF/UAS NF.

1

2

3

4

5

6

7

8

1

9

2

3

4

5

6

1

2

3

1

2

3

4

5

6

1

2

3

4

5

6

7

TA 5, USS 2

TA 2, USS 1

TA 3, USS 1

TA 4, USS 2

TA 1, USS 1

Border between USS 1 and USS 2

Border cells: TA1_8, TA1_9, TA2_6, TA3_2, TA3_3, TA4_1, TA4_2, TA4_3, TA4_6

UE location 1: TA3_1

UE location 2: TA3_2

UE location 3: TA5_7

Figure 6.10.2-2: Illustration of a UAV changeover scenario for its mobility from TA3_1 to TA5_7 via TA3_2.

Note that the serving USS may identify multiple potential USS(s) that may serve the UAV during its flight path, based on relevant criteria a serving USS need to select the most appropriate one for the UAV; and a USS may provide several flight paths (e.g., primary, secondary etc.) that can be used picked by a serving USS and reported to a UAV.

Even though pre-mission and in-mission flight planning may benefit from such multiple USS deployment, and as such, information related to the use cases have been used to illustrate the procedures. But it is not required to have such data to be available/needed.

Geographical area served by a USS is not restricted to the equivalent area served by an AMF.

Main principles as described in the procedures are as follows:

-	The serving USS subscribes to UAV location reporting events to AMF/GMLC/LMF;

-	If the serving USS/UTM detects that the UAV has left or is about to leave its service area, the USS/UTM may determine to trigger USS/UTM changeover to a new USS/UTM to serve the UAV;

-	The serving USS/UTM triggers the process of changeover of the UAV to the new USS/UTM (e.g. to assist in transferring the UAV context including e.g., subscribed events, UAS NF address, etc., how this is performed is out of 3GPP scope).

### 6.10.3	Procedures

Procedures for USS changeover in cases when a serving USS triggers communication with a target USS to jointly determine UAV’s flight path is shown in Fig. 6.10.3-1.

USS2

NEF2 (UAS NF2)

USS1

NEF1 (UAS NF1)

GMLC

NWDAF

AMF1

AMF2

UAV

1. PDU session establishment

2. Pre-mission flight planning request, incl. information about a flight destination

3. Determines information to request from 5GC, pre-selects suitable target USS-s, triggers communication with them to identify candidate border-crossing points for the UAV flight path planning purposes

4. Pre-mission flight planning assistance request

5. Determines services and NFs required to be invoked to provide the flight planning assistance

6. Steps 8 - 12 of the procedure for NEF-assisted pre-mission flight planning as shown in Figure 6.1.3.1-1

7. Namf_EventExposure_Subscribe request for UAV presence in the border cell/ TA

8. Pre-mission flight planning assistance response

9. Selection of the target USS and flight planning from a border cell/TA to a destination cell/TA

10. A target USS performs actions described in Steps 4 - 8

11. A target USS provides the serving USS information about UAV’s route planned from a border cell/TA to a destination cell/TA

12. Pre-mission flight planning response with the planned flight route

13. Namf_EventExposure_Notify about the UAV’s presence in the border cell/ TA from USS1 side

14. Namf_EventExposure_Subscribe/Notify for UAV deviation from the expected trajectory

15. Determining the flight direction of the UAV

16. Notification about the UAV leaving the area

17. Serving USS triggers the changeover to the target USS

18. Namf_EventExposure_Notify about the UAV’s presence in the border cell/ TA from USS2 side

19. Notification about the UAV entering the area

20. Informing the UAV about the new USS and re-invoking exposure services (if required) from the relevant NFs

21. Completion of the changeover between the serving USS and the target USS

Figure 6.10.3-1: Procedure for UAV changeover from one USS to another serving different geographical areas.

This procedure utilises Solution#1 of the NEF-assisted pre-mission flight planning as specified in clause 6.1 and includes the following main steps. Change of AMF(s) is not always required, both USS1 and USS2 may be served by the same AMF(s) during the entire UAV flight from the starting point to the destination point via the USS-determined flight path(s).

1.	The UAV (or UAV-C) establishes a PDU Session for communication with a serving USS (USS 1 in Fig. 6.10.3-1) as described in clause 5.2.3 of TS 23.256 [2].

2.	The UAV (or via its paired UAV-C) requests a Pre-mission flight planning service from the serving USS, if needed. The request message includes an identifier of the UAV (e.g. GPSI, CAA-Level UAV ID), information of the starting and destination points for the flight, requirements on the flight route (e.g. on time, shortest, highest, the farthest from a no-transmit zone) and may include candidate flight path(s) if available. Otherwise, continues in Step 3.

3.	USS 1 determines that the UAV-specified destination point lies outside a geographical area of its responsibility. Based on the pre-established knowledge (e.g. via pre-configuration, OAM, other proprietary means etc. that are not in scope of the study but assumed), the serving USS (USS 1) determines target USS (one or more) that may serve as a target USS. Once USS 1 pre-selects suitable target USS-s (USS 2 in Fig. 6.10.3-1), USS 1 triggers communication with each of the target USS to determines candidate border-crossing point(s) for the UAV.

NOTE 1:	A serving USS determines a target USS as well as a candidate border-crossing points between the serving USS and potential target USS-s by means outside 3GPP scope; for this purpose several criteria/metrics can be used, e.g. location of destination point, area that a specific USS serves, a number of UAVs in the area served by a specific USS, proximity to a no-transmit zone, USS load etc.

The serving USS in cooperation with the Uncrewed Aerial System Traffic Management (UTM) derives any other information (e.g. confidence level to be used by the NEF/UAS NF to determine likelihood that UAV will leave serving USS area) that can be used by 5GC to provide flight planning assistance information. This information from the serving USS/UTM, together with a target USS and candidate border-crossing points for the UAV, will be included in the Pre-mission flight planning assistance request that the serving USS will send to a serving NEF/UAS NF in Step 4.

NOTE 2:	The exact information that a USS can use for this purpose is out of 3GPP scope, however, it can contain for instance information about UAV capability, geographical coordinates of the candidate border-crossing point(s), indication to reconnect to the network once enter a cell/TA served by another USS, information about no-transmit zone (NTZ) etc.

4.	The USS sends a Pre-mission flight planning assistance request (e.g. Nnef_PreMissionFlighPlanning_Get or Nnef_PreMissionFlightPlanning_Subscribe) to the UAS NF/NEF with the information derived by the serving USS/UTM in Step 3 in addition to UAV’s identifier (e.g. GPSI), information of the starting point for the flight, a list of candidate border-crossing point(s), indication that the UAV will cross a USS border, and other parameters listed in Step 4 of clause 6.1.3.1 (e.g. requirements on the flight path, candidate flight path(s), accuracy level of predictions relevant to the flight planning).

5.	After receiving the Pre-mission flight planning assistance request from the serving USS, the NEF/UAS NF first translates/maps, if required, the parameters included in the USS request to 3GPP identifiers. For instance, the NEF determines a cell ID/tracking area identifier (TAI) of the cell/TA where the UAV-requested starting and destination points are located; similarly, if the request includes an indication that UAV will cross the USS-border and/or a list of the candidate border-crossing point(s) in form of geographical coordinates, the NEF maps them to a list of border cell IDs/TAIs.

NOTE 3:	Steps 4-5 may also be triggered without Pre-mission flight planning, actual messages and details are to be addressed during normative phase.

Next, the NEF determines the relevant NFs and specific service operations the NEF needs to invoke to collect the required information for the serving USS for UAV flight planning (e.g. NWDAF analytics service for Movement Behaviour analytics, GMLC service for Ranging/Sidelink Positioning location, AMF for UAV’s presence in bordering cells/TAs, AMF service for UAV’s deviation for the expected/assigned trajectory).

6.	The NEF/UAS NF invokes service operations towards the identified NFs as described in Steps 8-12 of the procedure for the NEF-assisted pre-mission flight planning in clause 6.1.3.1.

7.	For the UAV changeover purpose, if the NEF/UAS NF receives a list of candidate border-crossing points and/or the indication that the UAV will cross a USS border, the NEF/UAS NF identifies the AMF that serves NG-RAN nodes in all identified border cell(s)/TA(s); for that the NEF/UAS NF uses the Nnrf_NFDiscovery service from NRF. Once the AMF(s) information is retrieved, the NEF/UAS NF invokes an Namf_EventExposure_Subscribe request to subscribe to the UE/UAV’s presence in the area of interest wherein the area of interest is set to a cell ID/TAI of every identified border cell/TA.

8.	Once the NEF receives the relevant information from the determined 5GC NFs in Steps 3-7, the NEF/UAS NF responds to the Pre-mission flight planning assistance request from the USS 1 with the collected information. This information is for UAV’s flight path between the starting point and all candidate USS border-crossing points.

9.	Once the serving USS receives a response to its Pre-mission flight planning assistance request, USS 1 determines/selects a target USS from a list of pre-selected suitable USS in Step 3 and starts communication to request the target USS (USS 2 in Fig. 6.10.3-1) to perform the flight planning for the UAV from the border-crossing point(s) to the destination point in the geographical area served by USS 2.

10.	The target USS (USS 2) performs actions in Steps 4 – 8 to plan the UAV flight across its geographical area towards the UAV’s destination point.

11.	The target USS (USS 2) provides the serving USS (USS 1) information about UAV’s flight path(s) from a border cell(s)/TA(s) to the destination point (e.g., primary flight path, secondary/alternative flight path).

12.	Once the serving USS (USS 1) receives the information from the target USS (i.e., the flight path(s) for its part of the flight), the serving USS (USS 1) determines the primary, secondary etc. flight paths for the UAV from its starting point in USS 1 area to the destination point in USS 2 area; this includes all border-crossing points and cells/TAs.

The serving USS sends a response to the UAV (or its paired UAV-C) with the planned flight paths for the entire flight from the starting point to the destination point.

During the UAV flight (and to perform the changeover from USS 1 to USS 2):

13.	 Once the UAV enters a border cell/TA of the serving USS, the AMF notifies the NEF/UAS NF about the event (i.e., UAV enters the Area of Interest)

14.	If the NEF/UAS NF receives the notification about UAV entering a border cell/TA, the NEF/UAS NF may invoke additional services with the AMF to determine the UAV deviation from the expected trajectory (e.g. primary/secondary flight paths) and with the NWDAF for the Movement Behaviour analytics of the UE/UAV to determine whether the UAV will likely leave the USS area and continue following the flight path or not.

15-16.	If the NEF/UAS NF determines that the UE will likely leave (within the confidence level provided by the USS in Step 3) the geographical area served by the serving USS (USS 1), the NEF/UAS NF sends notification to USS 1 that the UAV will leave the area; for that purpose, the NEF/UAS NF may use a new service operation (e.g., Nnef_PreMissionFlightPlanning_InfoNotify or Nnef_PreMissionFlightPlanning_Notify) that can be send in response to the Nnef_PreMissionFlightPlanning_Get or Nnef_PreMissionFlightPlanning_Subscribe (assuming that they are used to request Pre-mission flight planning assistance in Step 4). Inside this notification, the NEF/UAS NF includes information about which border-crossing point will be used by the UAV; the NEF maps back the border cell ID/TAI triggering the event in step 14 to a specific border-crossing point from the list of candidate border-crossing points from Step 3.

17.	The serving USS (USS 1) communicates with the target USS (USS 2) to execute the changeover for the UAV; this communication and details are outside 3GPP scope, however, it is expected that a serving USS pass the target USS the information regarding which exposure services/notification are of relevance for the UAV, information about UAV’s identifiers (e.g. GPSI, CAA-Level UAV ID) as indicated in Step 2 or any other information is required by the target USS (or UAV itself) to establish the connection, see Step 1.

18-19.	The AMF serving a border cell/TA on USS2’s side (the AMF can be the same or different as on USS 1 side) notifies a serving NEF/UAS NF (can be the same or different from the NEF/UAS NF that communicates with USS 1) about the UAV presence in the border cell/TA from USS 2’s side.

The NEF/UAS NF notifies the USS 2 about this event using e.g. a Nnef_PreMissionFlightPlanning_InfoNotify request that can be introduced and used for this purpose.

20.	The source USS (USS 1) informs the UAV about the changeover to the new USS (USS 2) and, if required, informs the UAV that it needs to reconnect with the target USS (USS 2) and triggers the exposure services towards 5GC NFs similar to what the previous serving USS (USS 1) had before the changeover.

NOTE 4:	Steps 18-19 and 20 can continue in parallel and not intended to imply sequential processing.

21.	After receiving all the required information from the source USS (how this is done is outside 3GPP scope), the target USS responds to the Nnef_PreMissionFlightPlanning_InfoNotify request in such a way completing the changeover for the UAV.

### 6.10.4	Impacts on services, entities and interfaces

Impacts are similar as for Solution#1 (see clause 6.1.4) since this solution utilises pre-mission flight planning, if available. Additionality there may be other impacts such as:

AF/USS:

-	receiving notification from NEF when a UAV enters/leaves a border cell/TA under responsibility of the current serving USS,

-	providing information of suitable target USS candidates and candidate USS border-crossing points for UAV flight path(s),

-	requesting and responding to the pre-mission flight planning assistance request with the relevant information,

-	determining and providing information about the primary, secondary etc. flight paths for the UAV from its starting point in a geographical area served by a source USS to the destination point in a geographical area served by a target USS,

-	providing/requesting a confidence level to be used by the NEF/UAS NF to determine likelihood that a UAV will leave the geographical area serving by the USS.

NEF/UAS NF:

-	determining whether a UAV will leave / enter a border cell/TA; for that, the NEF/UAS NF functionalities need to be enhanced so that it can leave a geographical area served by USS,

-	communication with USS-s for pre-mission flight planning and triggering/completing changeovers from one USS to another, for instance, by sending an Nnef_PreMissionFlightPlanning_InfoNotify,

-	translating/mapping information about geographical areas served by USS-s to 3GPP identifiers (such as cell IDs and tracking area identifier (TAI)), specifically, some enhancements to map back a 3GPP identifier (e.g. border cell ID or TAI) to a specific border-crossing point information understandable by a USS,

-	sending/forwarding information to/from a USS with pre-mission flight planning assistance information. For that, enhancing responses (e.g., Nnef_PreMissionFlightPlanning_InfoNotify or Nnef_PreMissionFlightPlanning_Notify) to Nnef_PreMissionFlightPlanning_Get or Nnef_PreMissionFlightPlanning_Subscribe request that are being considered for requesting Pre-mission flight planning assistance.

## 6.11	Solution #11: Multi-USS Support in 5GC

### 6.11.1	Key Issue mapping

This solution addresses KI#1, “Enhancement of NEF services to support service exposure and interactions between MNOs and UTM functions”, focusing on the following aspect:

-	The scenario of multiple USS serving different geographical areas corresponding to the UAV flight path.

### 6.11.2	Description

The procedure for authentication and authorization of UAV by USS (UUAA) in TS 23.256 [2], clause 5.2 allows a particular USS to authenticate and authorize the UAV when it Registers or establishes a PDU Session. In current release a UAV is served by single USS for the duration of the connectivity between the USS and the UAV.

In this solution it is proposed to enhance the UUAA procedure such that the USS provides the UAS NF with a list of applicable serving USS info. UAS NF stores the list of serving USS info in the UUAA context and UAV UE store the information internally. It is also possible that list of serving USS info is preconfigured in UAS NF and UAV. The serving USS info also includes the serving area of the serving USS info. When a change of current serving USS occurs, the UAS NF is informed by the USS about the change. The UAS NF updates the UUAA context. UAS NF verifies whether a USS is authorized to initiate a change of USS based on the list of serving USS info in the UUAA context.

Editor’s note:	It shall be verified by SA3, whether the updating UUAA context in UAS NF from old USS to target USS can be updated without performing UUAA authentication and authorization.

### 6.11.3	Procedures

In 5GS, the following procedures are used for the authorization to support multiple USSs:

-	The UE indicates multi-USS capability in the service level container.

-	The procedure for UUAA-MM as specified in TS 23.256 [2], clause 5.2.2.2 with the following enhancements:

-	In step 2 and 3, an indication of multi-USS support is included, respectively by AMF and UAS NF;

-	In step 5, USS sends Naf_Authentication_AuthenticateAuthorize response message to UAS NF, which includes list of serving USS info (e.g., list of IP addresses, serving area information). The UAS NF stores list of serving USS info in the UUAA context;

NOTE:	How the serving USS acquire of information of other USSes is out of 3GPP scope.

-	In step 9, UAV receives NAS MM transport message forwarding authentication message from USS which includes the list of serving USS.

-	The procedure for UUAA-SM as specified in TS 23.256 [2], clause 5.2.3.2 with the following enhancements:

-	In step 1 and 2, an indication of multi-USS support is included, respectively by SMF and UAS NF;

-	In step 4, USS sends Naf_Authentication_AuthenticateAuthorize response message to UAS NF, which includes the list of serving USS info (e.g., list of IP addresses, serving area information;

-	The UAS NF stores list of serving USS info in the UUAA context;

-	In step 7, UAV receives Authorization Data which includes the list of serving USS.

UAV provides new USS information while triggering any PDU session signalling or registration signalling for UUAA.

In 5GS, the following procedures are used for the UAV tracking with support for multiple USSs:

The procedures for UAV Tracking as specified in TS 23.256 [2], clause 5.3 with the following enhancements:

-	UAV Location Reporting procedure in clause 5.3.2 and UAV presence monitoring procedure in clause 5.3.3, step 1, UAS NF checks that USS is in the list of serving USS info in the UUAA context;

-	Obtaining list of Aerial UEs in a geographic area procedure in clause 5.3.4, step 5, the UAS performs a filtering based on the list of serving USS in the UUAA context.

In 5GS, the following procedures are used for the change of current serving USS:

-	A procedure similar to USS initiated C2 pairing policy configuration in 5GS as specified in TS 23.256 [2], clause 5.2.5.4.1 with the following differences:

-	USS invokes Nnef_AFsessionWithQoS_Update service operation to the UAS NF for the PDU Session used for communication between the UAV and PCF. UAS NF checks that USS is in the list the serving USS in the UUAA context. The request (and following steps) includes the new serving USS IP address (instead of UAV-C pairing info/IP address);

-	UAS NF triggers Npcf_PolicyAuthorization_Update providing new serving USS IP information.

### 6.11.4	Impacts on services, entities and interfaces

UAS NF:

-	Receives multi-USS support indication from AMF/SMF during UUAA procedures.

-	Receives from USS and stores in UUAA context a list of serving USS info during UUAA procedures.

-	Authorizes USS requests (location tracking, change of USS/AF session configuration) based on the list of serving USS info in the UUAA context.

AMF/SMF

-	Sends multi-USS support indication to UAS NF during UUAA procedures.

UE

-	Provides multi-USS capabilities.

-	Receives list of serving USS info during UUAA procedures.

## 6.12	Solution #12: Support redundant connections for C2 communication reliability

### 6.12.1	Key Issue mapping

This solution addresses the C2 communication reliability aspect in key issues #1 “Enhancement of NEF services to support service exposure and interactions between MNOs and UTM function”.

### 6.12.2	Description

To achieve C2 communication reliability in some scenarios, e.g. UAV is hovering in area for some critical work, a UAV can establish two redundant C2 connections, which can be either with the same UAV-C or UTM. For redundant C2 connections with the same control entities, existing redundant transmission mechanism with different 5GS paths that is defined in clause 5.33.2 in TS 23.501 [5] can be reused. One C2 is selected as active C2 connection with the UAV-C or UTM, which is used for primary connection for the UAV control, while there can be another standby C2 connection established as redundancy and ready to take over the primary C2 connection when it goes down or other reasons deem necessary. The standby C2 connection may have different C2 traffic with different QoS policy than the active primary C2 connection. Establishing redundant C2 connections can be triggered or initiated by UAV or UTM. How to decide and select the redundant C2 connection is out of scope for 3GPP.

NOTE 1:	Redundant C2 connections may be used with two different PLMNs, but only redundant-C2-connection

within the same mobile network is in the scope of this release.

NOTE 2:	Using the existing redundant transmission defined in clause 5.33.2 in TS 23.501 [5] can address the End-to-End C2 reliability or N3/N9 C2 reliability. To achieve End-to-End C2 reliability, it’s assumed that NG-RAN

network supports dual connectivity, otherwise, only N3/N9 C2 reliability can be achieved.

This solution proposes to reuse existing redundant transmission, UAS network exposure functionality and procedures with only adding additional redundant information in C2 aviation payload and C2 authorization payload to allow UAV or UTM to synchronize information regarding the usage of redundant C2 connections via 5G system, because existing paring information within the aviation payload only include one C2 connection. Using this solution, UTM/USS and UAV can manage those different C2 connections accordingly to support UAV operations. This solution includes:

1.	UAV and 5G system support the redundant transmission mechanism defined in clause 5.33.2 in TS23.501 [5] to be able to establish two redundant C2 connection with a UAV-C or UTM within one network.

2.	This solution proposes to add C2 redundant information into the C2 aviation payload (from UAV to UTM) and C2 authorization payload (from UTM to UAV) to exchange C2 redundant information between UAV and UTM. The redundant information can include the indication of establishment of redundant C2 connections. This C2 redundant information can be part of the C2 paring information associate with the UAV-C.

It’s up to UAV and UTM to decide which C2 PDU session is primary or secondary. 5G system is only aware of two C2 redundant connections being established and has no knowledge of the primary and secondary C2 connection.

3.	The information exchange using C2 aviation payload and C2 authorization payload between UAV and UTM on the status change of C2 redundant connection can occur during C2 PDU connection establishment / modification phase, or UAV controller replacement phase.

Editor’s note:	Redundant C2 connections using same PDU session with N3/N9 redundancy is to be verified.

### 6.12.3	Procedures

#### 6.12.3.1	UE initiates PDU Session establishment for 2 redundant C2 communications with a UAV-C

Figure 6.12.3.1-1:	Procedure for PDU Session establishment for 2 redundant C2 communications

-	Step 0. The UAV has performed a successful UUAA with the USS.

-	Step 1. The UE first initiates PDU session establishment request for the redundant C2 connection with the UAV-C. In the PDU Session establishment request CAA-Level UAV ID, a C2 Aviation Payload to be used for C2 authorization which includes the C2 redundant information.

-	Step 2. The SMF then sends a Nnef_Authentication_AuthenticateAuthorize request including the C2 Aviation Payload, which is used to request authorization redundant C2 communication with 2 redundant C2 connections.

-	Step 3. The UAS NF/NEF forwards the received authorization request as a Naf_Authentication_AuthenticateAuthorize request to the UTM.

-	Step 4. The UTM performs C2 authorization based on the received information and sends the Naf_Authentication_AuthenticateAuthorize response to the UAS NF/NEF including the Service Level Device Identity (e.g. the CAA-Level UAV-ID), the C2 Authorization Result, and the C2 Authorization Payload (e.g. C2 redundant pairing information).

-	Step 5. The UAS-NF/NEF forwards the information received from the UTM in the Nnef_Authentication_AuthenticateAuthorize response sent to the SMF.

-	Step 6. SMF informs the UE about the C2 Authorization Result. SMF conducts PDU session establishment procedure until finalized for the establishment of two redundant C2 PDU connections for the UAV-C.

-	Step 7. [Conditional] If the C2 authorization is successful, the UTM subscribes via the UAS-NF to a PDU Session Status event for the PDU sessions used for the C2. The UAS NF determines DNN, S-NSSAI corresponding to the redundant PDU sessions used for the C2 communication and uses this DNN, S-NSSAI to subscribe to SMF for PDU Session Status event. The SMF detects, as described in steps 6-7 of figure 4.15.3.2.3-1 in TS 23.502 [3], when the PDU Session is established and send the PDU Session Status event report to the UTM via UAS NF/NEF including GPSI and two redundant C2 PDU session IP Addresses.

-	Step 8. (optional), The UTM stores the received two PDU session IP addresses and invokes the UTM initiated pairing policy configuration procedure (see figure 5.2.5.2.4-1) for the primary C2 connection.

-	Step 9. (optional), The UTM initiates C2 pairing policy configuration for the secondary C2 connection.

### 6.12.4	Impacts on services, entities and interfaces

USS/UTM:

-	Add C2 redundant information in C2 aviation payload and C2 authorization payload.

UE:

-	Need to support dual connectivity for End-to-End reliability.

## 6.13	Solution #13: Enhancement of NEF services for C2 communication assistance information

### 6.13.1	Key Issue mapping

This is a solution for Key Issue #1, in particular, this solution addresses whether and how to enhance NEF services to support service exposure and interactions between MNOs and UTM functions for supporting C2 communication reliability.

### 6.13.2	Description

This solution proposes to enhance NEF services and enable USS/UTM to request NEF assistance for C2 communication reliability. This solution enables UAV to decide to use or change to other C2 connectivity based on the C2 connectivity related information received from the USS/UTM such that C2 communication reliability (e.g. QoS requirements) cannot be sustained at specific location and time interval.

### 6.13.3	Procedures

Figure 6.13.3-1: Procedure for C2 communication reliability support assisted by NEF

1.	The UAV establishes a PDU Session for C2 communication as described in clause 5.2.3 of TS 23.256 [2].

2.	Authorization for C2 over Uu is performed as described in clause 5.2.5 of TS 23.256 [2] if not performed at step 1.

3.	The UAV requests C2 connectivity related information to the USS/UTM by using application layer message that is out of scope. The request message can include identifier of the UAV (e.g. GPSI, CAA-Level UAV ID). The request message can also include information of the flight route and flight start/end time for the UAV, and reporting mode (event triggered or periodically).

Flight route setup can be performed between the UAV and the USS/UTM, that is out of scope. As a result, the USS/UTM can know information of the flight route and flight start/end time for the UAV. The flight start time can be current time or sometime in the future.

4.	The USS/UTM derives information for the C2 connectivity related information request and decides to request assistance information from NEF. How the USS/UTM derives this information and the content to request assistance information derived at USS/UTM are out of scope.

The USS/UTM sends C2 connectivity assistance request to NEF. The request message includes identifier of the UAV (e.g. GPSI), information of the flight route and flight start/end time for the UAV (obtained in step 3), and reporting mode (event triggered or periodically).

NOTE 1:	USS/UTM can request C2 connectivity assistance information to NEF as internal decision although the UAV does not request C2 connectivity related information to the USS/UTM as described in step 3.

5.	The NEF maps parameters included in the request from the USS/UTM to information used by the 3GPP system (e.g. map the geographical area into an area of interest that is represented by a list of Cell IDs, gNB IDs or TAIs). The NEF determines services needed for the request, e.g. NWDAF analytics service (QoS Sustainability Analytics, Network Performance Analytics). Also, the NEF determines parameters used to request the analytics, e.g. QoS requirements, S-NSSAI related to C2 communication.

6.	If the NEF cannot provide the requested assistance information for C2 connectivity, the NEF responses to the USS/UTM to reject the C2 connectivity assistance request, may include the detail reason if available.

7.	If being rejected in step 6, the USS/UTM may response to the UAV to reject the C2 connectivity related information request. The response may include the detail reason if available.

8.	The NEF subscribes for notification on QoS Sustainability Analytics and Network Performance Analytics provided by NWDAF as defined in TS 23.288 [6]. For the subscription, appropriate parameters needed for these Analytics are provided to the NWDAF, e.g. Location information/AoI corresponding to the flight route for the UAV, Location information/AoI corresponding to the candidate flight route.

9.	The NEF generates assistance information for C2 connectivity based on the analytics from step 8. The assistance information may include the location information (e.g. waypoints/geographical areas that map to the Cell ID(s), TAI(s), gNB ID(s)) and time information to indicate that C2 communication reliability (e.g. QoS requirements) cannot be sustained at specific location and time interval.

10.	The NEF responses to the USS/UTM with the assistance information.

11.	The USS/UTM sends response to the UAV with the C2 connectivity related information, e.g. indication that C2 communication reliability cannot be sustained.

12.	Based on the C2 connectivity related information received from the USS/UTM, the UAV may decide to use or change to other C2 connectivity, e.g. use Standby C2 connectivity as Active, use C2 connectivity over EPS.

NOTE 2:	USS/UTM are used to represent the consumer of the enhanced NEF services in Figure 6.13.3-1, the consumer can also be other entities, e.g. TPAE.

### 6.13.4	Impacts on services, entities and interfaces

UAS NF/NEF:

-	Enhance for supporting C2 connectivity assistance service.

USS/UTM:

-	Enhance for providing C2 connectivity related information to UAV.

## 6.14	Solution #14: C2 Communication Reliability with Redundant User Plane Paths

### 6.14.1	Description

The study aspect on whether and how to enhance NEF services to support service exposure and interactions between MNOs and UTM functions for supporting C2 communication reliability is contained in KI#1. This solution proposes to reuse the redundant user plane paths introduced in TS 23.501 [5] Annex F with changes and enhancements as described in clause 6.14.2. The solution applies in below scenario, same as in TS 23.501 [5] Annex F, with UE replaced with UAV:

-	A UAV with multi-SIM single operator: the UAV to set up redundancy C2 connections to UTM.

-	The UAV is in an area where RAN coverage is redundant: the UAV in the area can connect to multiple gNBs from the same location. The selection of gNBs can be distinct from each other.

The solution is mainly reusing the existing mechanism and has no impact on RAN specifications.

UAV application layer handles the redundant connections and how this is done is outside of 3GPP scope (e.g. how traffic is sent over one, both, or switched is outside the scope and up to the application layer).

Editor’s Note:	Any implications or assumptions on the UAV ID use in multiple SIM single operator needs to be verified.

### 6.14.2	Procedures

This clause describes an approach to realize C2 communication reliability with multiple user plane paths. The approach assumes a RAN deployment where redundant coverage by multiple gNBs (in the case of NR) is generally available, and a UAV can connect to multiple gNBs independently.

The UAV with redundant connections with gNBs requests the establishment of PDU Sessions that use independent RAN and CN network resources using the mechanisms outlined below.

This deployment option has a number of preconditions:

-	The redundancy framework uses separate gNBs to achieve user plane redundancy over the 3GPP system. It is however up to operator deployment and configuration whether separate gNBs are available and used.

-	The UAV can connect to different gNBs independently.

-	RAN coverage is redundant in the target area: it is possible to connect to multiple gNBs from the same location. To ensure that the UAV connects to different gNBs, the gNBs need to operate such that the selection of gNBs can be distinct from each other (e.g. gNB frequency allocation allows the UAV to connect to multiple gNBs).

-	The core network UPF deployment is aligned with RAN deployment and supports redundant user plane paths.

-	The underlying transport topology is aligned with the RAN and UPF deployment and supports redundant user plane paths.

-	The physical network topology and geographical distribution of functions also supports the redundant user plane paths to the extent deemed necessary by the operator.

-	The operation of the redundant user plane paths is made sufficiently independent, to the extent deemed necessary by the operator, e.g. independent power supplies.

Figure 6.14.2-1 illustrates the architecture view. The UAV is connected to gNB1 and gNB2, respectively and connection1 sets up a PDU Session via gNB1 to UPF1, while connection2 sets up a PDU Session via gNB2 to UPF2. UPF1 and UPF2 connect to the same UAV control entity (UTM). UPF1 and UPF2 are controlled by SMF1 and SMF2, respectively.

Figure 6.14.2-1: Architecture with redundancy connections between UAV and control entities

The approach comprises the following main components.

-	gNB selection: The selection of different gNBs for the UAV is realized by the concept of UAV Reliability Groups for the connections of UAV and also for the cells of gNBs. By grouping the connections of UAV and cells of gNBs in the network into more than one reliability group and preferably selecting cells in the same reliability group as the connection, it is ensured that connections of UAV can be assigned different gNBs for redundancy as illustrated in Figure 6.14.2-2, where connection1 and the cells of gNB1 belong to reliability group A, and connection2 and the cells of gNB2 belong to reliability group B.

Figure 6.14.2-2: Reliability group-based redundancy concept in RAN

For determining the reliability grouping of a UE, one of the following methods or a combination of them can be used as described in TS 23.501 [5] Annex F:

-	It could be configured explicitly to the UE and sent in a Registration Request message to the network using an existing parameter (such as an S-NSSAI in the Requested NSSAI where the SST is UAS; the Reliability Group can be decided by the SD part).

-	It could also be derived from existing system parameters (e.g. SUPI, PEI, S-NSSAI, RFSP) based on operator configuration.

The Reliability Group of each UE is represented via existing parameters and sent from the AMF to the RAN when the RAN context is established, so each gNB has knowledge about the reliability group of the connected UEs.

NOTE:	An example realisation can be as follows: the UE's Allowed NSSAI can be used as input to select the RFSP index value for the UE. The RAN node uses the RFSP for RRM purposes and can based on local configuration determine the UE's Reliability Group based on the S-NSSAI in Allowed NSSAI and/or S-NSSAI for the PDU Session(s).

The reliability group of the RAN (cells of gNBs) entities are pre-configured by the O&M system in RAN. It is possible for gNBs to learn the reliability group neighbouring cells as the Xn connectivity is set up, or the reliability group of neighbouring cells are also configured into the gNBs.

The Reliability Group of each connection is represented via existing parameters and sent from the AMF to the RAN when the RAN context is established, so each gNB has knowledge about the reliability group of the connections.

The reliability group of the RAN (cells of gNBs) entities are pre-configured by the O&M system in RAN. It is possible for gNBs to learn the reliability group neighbouring cells as the Xn connectivity is set up, or the reliability group of neighbouring cells are also configured into the gNBs.

If the UAV is moving out of the coverage of cells of its current connected reliability groups or link quality is below a given threshold, the connections may be handed over to cells in another reliability groups, the gNB initiates the handover to cells in the appropriate reliability groups whenever such suitable cells are available.

### 6.14.3	Impacts on services, entities and interfaces

UAV:

-	Support redundancy connections to same control entities with multiple gNBs.

AMF:

-	Enhanced to enable reliability group for multiple connections of UAV with multiple gNBs, reusing existing parameters can mitigate that AMF does not identify redundant connections.

## 6.15	Solution #15: NTZ restriction Provisioning aspects

### 6.15.1	Key Issue mapping

This solution aims to resolve Key Issue #3, " Support of No Transmit Zones ".

### 6.15.2	Description

Besides preconfiguration, this solution addresses the dynamical update of NTZ policies.

### 6.15.3	Procedures

#### 6.15.3.1	NTZ Provisioning

The call flow below is based on clause 4.15.6.7 Service-specific parameter provisioning in TS 23.502 [4].

NOTE 1:	The UE can be preconfigured with NTZ polices and updates can be applied based on the procedure below.

The procedure can also be delta update.

Figure 6.15.3.1-1 NTZ Parameter provisioning

1.	To create a new request, the USS/TPAE invokes a Create / Update / Delete NTZ parameters’ service operation (or reuse any existing NEF APIs) to UAS NF to request. The request may include subscription information to the report on the UE(NTZ) policy delivery outcome.

To update or remove an existing request, the USS/TPAE invokes an NTZ parameter Update or Delete service operation together with the corresponding Transaction Reference ID which was provided to the USS/TPAE in Create response message.

2.	The USS/TPAE sends its request to the NEF/UAS-NF. The NEF/UAS-NF authorizes the USS/TPAE request. The NEF/ UAS-NF performs the following mappings:

2a.	(Optional) The NEF/UAS-NF may invoke Nudm_SDM_Get service operation to perform the following mappings:

-	Map the GPSI in Target UE Identifier into SUPI, according to information received from UDM.

-	Map the External Group Identifier in Target UE Identifier into the Internal Group Identifier, according to information received from UDM. (e.g. 3GPP UAV ID, CAA Level UAV ID).

If the USS/TPAE subscribed to the outcome of UE Policy delivery, the USS/TPAE indicates where (resource URI of the target recipient of the notification) the USS/TPAE receives the corresponding notifications.

2b.	 (Optional) (in the case of Create or Update): The NEF/UAS-NF may need to authorize the service-specific parameter provisioning request with the UDM by sending a Nudm_ServiceSpecificAuthorisation_Create service operation as the USS/TPAE entity is outside the 3GPP domain.

(in the case of NTZ parameter delete): The NEF/UAS-NF requests the UDM to remove the authorization of the service-specific parameters provisioned by sending a Nudm_ServiceSpecificAuthorisation_Remove service operation.

3.	(in the case of NTZ parameters’ Create or Update): The NEF/UAS-NF stores the USS/TPAE request information in the UDR as the "UEPolicySet" (Data Subset setting to " NTZ Info") together with the assigned Transaction Reference ID.

(in the case of NTZ parameters’ Delete): The NEF/UAS-NF deletes the USS/TPAE request information from the UDR.

4.	The NEF/UAS-NF responds to the USS/TPAE. In the case of Create response message, the response message includes the assigned Transaction Reference ID. If the UE is registered to the network and the PCF performs the subscription to notification to the data modified in the UDR by invoking Nudr_DM_Subscribe (USS/TPAE NTZ parameter provisioning information, SUPI, Data Set setting to "UEPolicySet", Data Subset setting to " NTZ Info") at step 0, the following steps are performed:

5.	The PCF(s) receive(s) a Nudr_DM_Notify notification of data change from the UDR.

NOTE 2:	PCF does not have to subscribe for each UE the application specific information, e.g. if PCF has already received the application specific information for a group of UE or PLMN. The same NTZ specific information is delivered to every UE in a group.

6.	The PCF initiates UE Policy delivery (as specified in clause 6.15.3.2).

7.	(Optional) If the USS/TPAE subscribed to notifications about the outcome of UE Policies delivery due to NTZ parameter provisioning targeting a single UE and the PCF is notified of UE Policy Container from the AMF, the PCF notifies the UE Policy delivery result contained in the UE Policy container as the outcome of the procedure to NEF/UAS-NF by sending Npcf_EventExposure_Notify.

If the PCF is notified about UE Policy delivery failure from the AMF due to UE not reachable, the PCF may determine to retry step 6 of this procedure when the UE becomes reachable. In such a case, the PCF may report the interim status i.e. UE is temporarily unreachable as the outcome of the procedure to NEF/UAS-NF by sending Npcf_EventExposoure_Notify.

If the PCF determines the failure of the UE Policies delivery procedure, the PCF notifies the failure with an appropriate cause such as UE is unreachable as the outcome of the procedure to NEF/UAS-NF by sending Npcf_EventExposure_Notify.

8.	(Optional) When the NEF/UAS-NF receives Npcf_EventExposure_Notify, the NEF/UAS-NF performs information mapping (e.g. AF(USS/TPAE) Transaction Internal ID provided in Notification Correlation ID to AF(USS/TPAE) Transaction ID, SUPI to GPSI, etc.) and triggers the appropriate Nnef_ServiceParameter_Notify message.

#### 6.15.3.2	UE Configuration Update procedure for transparent UE Policy delivery

This procedure is initiated when the PCF wants to update UE policy information (i.e. UE policy) in the UE configuration. The below call flow procedure follows the steps from clause 4.2.4.3 in TS 23.502 [4]. The UE policy delivery mechanism is reused for NTZ policy delivery.

#### 6.15.3.3	No-transmission zone parameters & UE behaviour

The NTZ policy information will be based on the coordinates and the indication flag whether NTZ is applicable, as well as the list of allowed frequency bands. The below tables are described as an example. More details on the data structure need to be defined by stage3.

Editor's note:	This clause represents an example of NTZ parameters. The detailed and complete parameters are FFS based on provisioning data.

Table 6.15.3.3-1: Sample NTZ Parameters provided by USS/TPAE

| NTZ parameters | Description |
| --- | --- |
| Geographical area | Identifies the geographical area indicated by the geographical area field (including coordinates with (longitude, latitude & height) (similar data types can be applied for height-dependent NTZ when served by RAN) |
| Radio Parameters | Radio parameters, e.g., which bands are allowed and whether the transmission is restricted for UE in a certain geography |

This solution assumes that the UE, upon applying UE Policy (NTZ policy) with NTZ information, implements the following behaviour:

1.	UE does not do cell search or any radio transmission in a no-transmission zone.

2.	UE does not use the restricted bands in band-restricted geography.

Editor's note:	When NTZ band restrictions information is configured, how UE handles RFSP is FFS.

### 6.15.4	Impacts on services, entities and interfaces

UE

-	Complying to new NTZ information based on policy information.

NEF

-	Supporting new API/new parameters to support NTZ provisioning.

UDR

-	Storage of new NTZ information.

PCF

-	Support of new NTZ policy.

## 6.16	Solution #16: NTZ Enforcement in 5GC

### 6.16.1	Key Issue mapping

This solution addresses KI#3, “Support of No Transmit Zones”.

### 6.16.2	Description

This solution assumes that NTZ enforcement is provided to the 5GC from USS dynamically. This solution is intended for 5GC to enforce NTZ information for the UAV dynamically so that UAV does not transmit in the NTZ after receiving NTZ enforcement.

In this solution, the NTZ enforcement request is initiated from the USS/UTM and sent to the UAS-NF in 5GC. The request includes the designated NTZ information (areas, spectrum, etc.) and target UAV identifiers (or all UAVs). The UAS NF maps the NTZ areas to network areas and locates the serving AMFs of the affected UAVs and forwards the NTZ enforcement request to the serving AMFs.

The serving AMF may perform the following actions:

-	Use the location service to determine whether the UAV is in or out of the NTZs.

-	For those in the NTZs and in Connected mode, inform the SMF to deactivate the PDU sessions of the UAV (C2 communication may be exempted), and inform the RANs to not schedule UL transmissions for those UAVs.

Editor's Note:	It is FFS whether and how to enforce NTZ for UAVs in Idle state.

### 6.16.3	Procedures

Figure 6.16.3-1: NTZ enforcement in 5GC

1.	The UAS NF receives the NTZ enforcement request from the USS/UTM. The request includes the NTZ information and affected UAV Identifiers (or all UAVs). The NTZ information includes one or multiple NTZ areas, frequency bands that UAVs are not allowed to transmit, etc.

2.	The UAS NF query UDM and may use the UAV tracking mechanism to determine the location of the affected UAVs, whether they are in NTZs, and locate their serving AMFs.

Editor’s Note:	It is FFS whether UAS NF needs to use the UAV tracking mechanism to determine the location of the affected UAV.

NOTE 1:	This solution assumes that the affected UAVs have successfully registered at the network.

Editor’s Note:	It is FFS how to enforce NTZ to UAVs not yet registered and without NTZ information.

The following steps are repeated for each affected UAV.

3.	The UAS NF forwards the NTZ enforcement request to the serving AMFs.

4.	The AMF may further use location service to determine the location of the affected UAVs.

If the affected UAV is in Connected mode,

5.	The AMF may request the SMF to deactivate the resource for the PDU Sessions of the affected UAV.

6.	The AMF also notifies the affected UAV of the NTZ enforcement.

NOTE 2:	If the affected UAV is in NTZ zone, step 6 performed only when downlink data reception in NTZs is allowed given the regulatory requirements are not violated. And the affected UAV shall not respond to the notification.

7.	The AMF may also send the serving RAN a NTZ enforcement request with NTZ info (e.g., NTZ area and restricted freq. band).

8.	Based on RAN’s discretion, the RAN may perform NTZ enforcement for the affected UAV (e.g. release the RRC connection of the UAV, etc).

Editor’s Note: RAN aspect is not in scope of SA2 and will be based on RAN decision.

Editor’s Note: Whether and how the height information is incorporated, which will ensure UAV does not trigger any operation beyond the mentioned height, is FFS. RAN enforcement for height in the NTZ is FFS.

### 6.16.4	Impacts on services, entities and interfaces

UAS NF:

-	Supports handling of NTZ enforcement request, including forwarding the request to other NFs (e.g. AMF).

AMF

-	Supports notification of NTZ enforcement/restriction to the affected UAVs using NAS signalling.

-	Supports requesting the SMF to enforce the NTZ (e.g., deactivate PDU Sessions).

-	Supports requesting the RAN to enforce the NTZ (e.g., not scheduling UL transmission).

-	Reject service request for the reason of NTZ control.

RAN

-	Supports NTZ enforcement (e.g., not scheduling UL transmission, release RRC).

UAV

-	Supports NTZ enforcement.

## 6.17	Solution #17: Support of No Transmit Zone

### 6.17.1	Key Issue mapping

This solution addresses KI#3.

### 6.17.2	Description

This paper proposes solution for Aerial UEs/UAVs and 5GC/EPC to ensure the support for No Transmit Zone. The following are the main principles of the solution:

-	The No Transmit Zone is a kind of restricted area for UAVs not to avail any services from the network. Therefore, this No Transmit Zone information, which may consist of one or more restricted cells on a certain frequency band. These information need to be made available to UAVs and 5GC/EPC network.

-	As only UAVs will be restricted in No Transmit Zone and normal UEs can operate, hence this information will be applicable per subscription at UDM/UDM+HSS. UAVs shall be pre-configured with this No Transmit Zone information. If the UAVs have provided capability indication of supporting No Transmit Zone then AMF shall provide with updated No Transmit Zone information during Registration Accept and UE configuration update procedure.

-	Using available No Transmit Zone information, UAV shall not select/reselect any restricted cells.

-	To address the connected mode mobility, AMF shall provide the No Transmit Zone information to NG-RAN when UAV does registration with the network. Source NG-RAN shall not handover the UAV to a target NG-RAN node if the target is a restricted cell as per the available NTZ information which has been received from AMF. If the target cell is a restricted cell as per NTZ information, target NG-RAN shall reject the N2 based handover procedure.

Editor's note:	The NG-RAN action for the handover for UAV from non-restricted cell to restricted cell is dependent on RAN WGs.

-	If AMF receives the registration request from Aerial UE which does not indicate the capability to support the No Transmit Zone and the UAV is inside the No Transmit Zone then AMF shall reject the registration with existing cause code #15 “no suitable cells in tracking area” as specified in TS 24.501 [10].

Editor's note:	Whether and how the height information is incorporated, which will ensure UAV does not trigger any operation beyond the mentioned height, is FFS. RAN enforcement for height in the NTZ is FFS.

### 6.17.3	Procedures

The procedure of ensuring support of No Transmit Zone by Aerial UE and network.

Figure 6.17.3-1:	 Procedure for NTZ support information

1a.	UAV is pre-configured with NTZ information.

1b.	UDM is provisioned with NTZ information for the UAV.

2.	UAV sends Registration Request with capability indication for support of NTZ.

3.	AMF gets the subscriber data information from the UDM.

4.	AMF sends the registration Accept with updated NTZ information.

### 6.17.4	Impacts on services, entities and interfaces

This solution may have the following impacts to existing entities and interfaces:

UAV:

-	will send its capability indication of supporting NTZ.

-	will enforce of cell selection/reselection with respect to available NTZ information.

AMF:

-	will provide updated NTZ information in registration accept and UE configuration update procedure

-	will provide NTZ information for the Aerial UE to NG-RAN

NG-RAN:

-	Source NG-RAN shall not handover the Aerial UE to a target NG-RAN node if the target is a restricted cell as per the available NTZ information which has been received from AMF.

-	If the target cell is a restricted cell as per NTZ information, target NG-RAN shall reject the N2 based handover procedure.

UDM:

-	will be provisioned with NTZ information for the UAVs.

# 7	Overall Evaluation

Editor's note:	This clause provides evaluations of different solutions, if exists.

# 8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

Annex A:
Background Information about No Transmit Zones

# A.1	CEPT Decision 22(07)

This is an extract of the CEPT Decision 22(07) for No Transmit Zones.

In November 2022, CEPT made Decision 22(07) on Harmonised technical conditions for the usage of aerial UE for communications based on LTE and 5G NR in several bands harmonized for MFCN. The decision assumes multiple technical conditions and requirements to support aerial UEs in mobile systems (both LTE and NR). Two notable ones are no-transmit zone (NTZ) and out-of-band emission (OOBE) requirements, as shown in the following excerpt from the Decision (further details are in the Appendix):

In addition to the already harmonised technical conditions for MFCN bands and for spectrum compatibility purposes, there is the need to define some spectrum operational restrictions. This can be done using "no-transmit zones", which should be defined at national level as a geographical area where aerial UE are not allowed to operate in a certain frequency band. Another measure to achieve coexistence is to define additional OOB emission limits specific to aerial UE (to avoid interference to other services in some other bands (e.g. to protect MetSat at 1675-1710 MHz). The requirement may apply to aerial UE according to their operational frequency band, e.g. aerial UE operating in a specific band or specific channel (see no-fly zone definition set out in ECC Report 309, in this Decision referred to as "no-transmit zone"). In some cases, operation of aerial UE also requires respective cross-border coordination agreements. . . . ECC DECISION OF 18 NOVEMBER 2022 ON HARMONISED TECHNICAL CONDITIONS FOR THE USAGE OF AERIAL UE FOR COMMUNICATIONS BASED ON LTE AND 5G NR IN THE 703-733 MHZ, 832-862 MHZ, 880-915 MHZ ,1710-1785 MHZ, 1920-1980 MHZ, 2500-2570 MHZ AND 2570-2620 MHZ MFCN HARMONISED BANDS (ECC DECISION (22)07) "The European Conference of Postal and Telecommunications Administrations, Considering . . . l) that a no-transmit zone in this Decision is defined as a geographical area where aerial UE are not allowed to transmit for spectrum compatibility purposes in a given harmonised MFCN band or part of it; m) that national studies are needed, as appropriate, to define no-transmit zones for spectrum compatibility purposes, for aerial UE operating in the relevant frequency bands; n) that a mechanism is necessary to ensure that aerial UE respect no-transmit zones; . . . DECIDES . . . that no-transmit zones as described in this Decision should be defined and implemented at national level and where necessary coordinated with neighbouring countries;

Some further details on no-transmit zones from the ECC Decision 22(07):

A1.2 OPERATIONAL CONDITIONS The operational conditions to be defined and implemented at national level provide additional measures to the technical conditions in order to protect other services. 703-733 MHz: Protection of DTT receivers and RAS sites Aerial UE operating in 703-733 MHz should not transmit when less than 30 m above ground level to avoid interference to DTT receivers; Nationally determined no-transmit zones are required around RAS sites operating in 1400-1427 MHz for aerial UE operating in the 703-718 MHz frequency band, as appropriate. 832-837 MHz: Protection of RAS sites Nationally determined no-transmit zones are required around RAS sites operating in 1660-1670 MHz for aerial UE operating in the 832-837 MHz frequency band, as appropriate. 2500-2570 MHz/2570-2620 MHz: Protection of RAS sites and radars Nationally determined no-transmit zones are required around RAS sites operating in 2690-2700 MHz for aerial UE operating in the 2500-2570 MHz or 2570-2620 MHz frequency band, as appropriate; Nationally determined no-transmit zones might be required around radars operating in 2700-2900 MHz for aerial UE operating in the 2500-2570 MHz or 2570-2620 MHz frequency band.

Annex B:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-01 | SA2#160e-adhoc | S2-2400226 | - | - | - | First version of the TR skeleton produced | 0.0.0 |
| 2024-01 | SA2#160-Ad Hoc-e | S2-2400169, S2-2401814, S2-2401815, S2-2401816, S2-2401817, S2-2401818, S2-2401819 | - | - | - | Approved pCRs implemented | 0.1.0 |
| 2024-03 | SA2#161 | S2-2403270, S2-2403694, S2-2403846, S2-2403273, S2-2403274, S2-2403695, S2-2403697, S2-2403840, S2-2403699 | - | - | - | Approved pCRs implemented Some editorial changes by rapporteurs -Added RTA in the acronym list -To use inclusive terminologies (i.e. to avoid non-inclusive terminologies) according to TR 21.801 - In Sol#4: modify unmanned to uncrewed, manned to crewed - In Sol#7: modify blacklist to blocklist, whitelist to allowlist -Added reference to 23.287 -Corrected many formatting errors and clause numbering | 0.2.0 |
| 2024-04 | SA2#162 |  | - | - | - | Approved pCRs implemented. Editorial changes by rapporteurs: format fixes, adding missing titles, figure titles, fixing spelling mistakes etc. S2-2405537, S2-2405538, S2-2405806, S2-2405542, S2-2405543, S2-2405544, S2-2405545, S2-245547, S2-2405808, S2-2405807. | 0.3.0 |