| 3GPP TR 23.700-81 V0.3.0 (2022-05) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study of Enablers for Network Automation for 5G 5G System (5GS); Phase 3 (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	9

1	Scope	11

2	References	11

3	Definitions of terms and abbreviations	12

3.1	Terms	12

3.2	Abbreviations	12

4	Architectural Assumptions and Principles	12

5	Key Issues	12

5.1	Key Issue #1: How to improve correctness of NWDAF analytics	12

5.1.1	Description	12

5.2	Key Issue #2: NWDAF-assisted application detection	13

5.2.1	Description	13

5.3	Key Issue #3: Data and analytics exchange in roaming case	14

5.3.1	Description	14

5.4	Key Issue #4: How to Enhance Data collection and Storage	14

5.4.1	Description	14

5.5	Key Issue #5: Enhance trained ML Model sharing	14

5.5.1	Description	14

5.6	Key issue #6: NWDAF-assisted URSP	15

5.6.1	Description	15

5.7	Key Issue #7: Enhancements on QoS Sustainability analytics	15

5.7.1	Description	15

5.8	Key Issue #8: Supporting Federated Learning in 5GC	16

5.8.1	Description	16

5.9	Key Issue #9: Enhancement of NWDAF with finer granularity of location information	17

5.9.1	Description	17

5.10	Key Issue #10: Interactions with MDAS/MDAF	17

5.10.1	Description	17

6	Solutions	18

6.0	Mapping Solutions to Key Issues	18

6.1	Solution #1: Improving Correctness using multiple ML models for an analytics report	20

6.1.1	Description	20

6.1.2	Procedures	21

6.1.2.1	General	21

6.1.2.2	Support for describing multiple ML models in provisioning service operations	21

6.1.3	Impacts on existing nodes and functionality	22

6.2	Solution #2: Improving the Correctness of Service Experience Predictions with Contribution Weights	22

6.2.1	Description	22

6.2.2	Procedures	23

6.2.3	Impacts on existing nodes and functionality	23

6.3	Solution #3: Accuracy based NWDAF Analytics Correctness Improvement	23

6.3.1	Description	23

6.3.2	Procedures	24

6.3.3	Impacts on services, entities and interfaces	26

6.4	Solution #4: Determining ML model drift for improving analytics accuracy	26

6.4.1	Description	26

6.4.2	Procedures	27

6.4.3	Impacts on services, entities and interfaces	29

6.5	Solution #5: Enhancements on ML model provision to improve correctness of NWDAF analytics	29

6.5.1	Description	29

6.5.2	Procedures	30

6.5.2.1	Procedures for AnLF based error monitoring	31

6.5.2.2	Procedures for MTLF based error monitoring	32

6.5.2.3	Procedures for MTLF to detect dynamics on the analytics	33

6.5.3	Impacts on existing nodes and functionality	34

6.6	Solution #6: Correctness improvement of NWDAF by determining ML model performance	34

6.6.1	Description	34

6.6.2	Procedures	35

6.6.3	Impacts on existing nodes and functionality	36

6.7	Solution #7: Enhancements to NWDAF analytics services	36

6.7.1	Description	36

6.7.2	Procedures	37

6.7.2.1	Analytics consumer rating an NWDAF analytics provided by an NWDAF containing AnLF	37

6.7.2.2	NWDAF containing AnLF rating an ML model provided by an NWDAF containing MTLF	38

6.8	Solution #8: NWDAF assisted service type detection	39

6.8.1	Description	39

6.8.2	Procedures	42

6.8.3	Impacts on existing nodes and functionality	42

6.9	Solution #9: NWDAF-assisted application detection	43

6.9.1	Description	43

6.9.2	Input Data	43

6.9.3	Output Analytics	44

6.9.4	Procedures	45

6.9.5	Impacts on existing nodes and functionality	45

6.10	Solution #10: Support for Data and Analytics Exchange in Roaming Case	46

6.10.1	Description	46

6.10.2	Procedures	47

6.10.3	Impacts on services, entities and interfaces	49

6.11	Solution #11: PDU session management in roaming scenarios using network analytics	49

6.11.1	Description	49

6.11.2	Procedures	50

6.11.3	Impacts on existing nodes and functionality	51

6.12	Solution #12: DCCF and MFAF Relocation	51

6.12.1	Description	51

6.12.2	Procedures	52

6.13	Solution #13: NWDAF MTLF and NWDAF AnLF interoperability support for registration and discovery in 5GC	53

6.13.1	Description	53

6.13.2	Procedures	54

6.14	Solution #14: Enhance trained ML Model sharing via ML Model format	55

6.14.1	Description	55

6.14.1.1	General	55

6.14.1.2	Registration and discovery of NWDAF (containing MTLF) with ML model Format Information	56

6.14.3	Impacts on existing nodes and functionality	57

6.15	Solution #15: ML model sharing with different AnLF providers	57

6.15.1	Description	57

6.15.2	Procedures	58

6.15.3	Impacts on existing nodes and functionality	58

6.16	Solution #16: NWDAF assisted URSP determination	58

6.16.1	Description	58

6.16.1.1	Input Data	60

6.16.1.2	Output Analytics	61

6.16.2	Procedures	62

6.16.3	Impacts on existing nodes and functionality	62

6.17	Solution #17: NSSP in roaming scenarios using network analytics	63

6.17.1	Description	63

6.17.2	Procedures	63

6.17.3	Impacts on existing nodes and functionality	64

6.18	Solution #18: Integrity KPI for QoS Sustainability Analytics	64

6.18.1	Key Issue mapping	64

6.18.2	Description	64

6.18.3	Procedures	66

6.18.4	Impacts on services, entities and interfaces	67

6.18.4.1	Input data	67

6.18.4.2	Output analytics	67

6.19	Solution #19: Enhanced QoS Sustainability Analytics in the finer granularity area	68

6.19.1	Description	68

6.19.2	Procedures	68

6.19.2.1	General	68

6.19.2.2	Input data	68

6.19.2.3	Output analytics	70

6.19.2.4	Procedures	71

6.19.3	Impacts on existing nodes and functionality	71

6.20	Solution #20: GTP metrics for QoS Sustainability analytics	71

6.20.1	Description	71

6.20.2	Input data	72

6.20.3	Procedures	72

6.20.3	Impacts on services, existing nodes and functionality	72

6.21	Solution #21: Federated Learning procedure between different NWDAFs	73

6.21.1	Description	73

6.21.2	Procedures	73

6.21.2.1	NWDAF "FL Capability" Registration	73

6.21.2.2	NWDAF Selection for FL operation	73

6.21.2.3	FL operation between NWDAFs containing MTLF	74

6.21.3	Impacts on existing nodes and functionality	76

6.22	Solution #22: Federated learning group creation	76

6.22.1	Description	76

6.22.2	Procedures	77

6.22.3	Impacts on existing nodes and functionality	78

6.23	Solution #23: Support of federated learning for model training	78

6.23.1	Description	78

6.23.2	Procedures	80

6.23.3	Impacts on services, entities and interfaces	83

6.24	Solution #24: Horizontal Federated Learning among Multiple NWDAFs	83

6.24.1	Description	83

6.24.2	Procedures	84

6.24.2.1	General procedure for Federated Learning among Multiple NWDAF Instances	84

6.24.2.2	Procedure for usage of Federated Learning in Abnormal Behaviour	86

6.24.3	Impacts on existing nodes and functionality	87

6.25	Solution #25: Outdoors Advertisement use case with finer granularity location information	87

6.25.1	Description	87

6.25.2	Input Data	88

6.25.3	Output Analytics	88

6.25.4	Procedures	88

6.25.5	Impacts on existing nodes and functionality	90

6.26	Solution #26: Finer granularity of location information based on cell sequence	90

6.26.1	Description	90

6.26.2	Procedures	90

6.26.3	Impacts on existing nodes and functionality	91

6.27	Solution #27: Relative Proximity Analytics	91

6.27.1	Description	91

6.27.2	Procedures	93

6.27.3	Impacts on existing nodes and functionality	94

6.28	Solution #28: Detect and Improve correctness of NWDAF analytics	95

6.28.1	Description	95

6.28.2	Procedures	97

6.28.3	Impacts on Existing Nodes and Functionality	98

6.29	Solution #29: Detection of ML Model degradation and actions	99

6.29.1	Description	99

6.29.2	Procedures	99

6.29.2.1	General	99

6.29.2.2	Detection of ML Model Degradation	100

6.29.2.3	Act Upon ML Model Degradation	100

6.29.2.4	Procedure for the registration of ML model monitoring	100

6.29.2.5	Procedure for the detection of ML model degradation	101

6.29.2.6	Procedures for ML Model Provisioning	103

6.29.3	Impacts on Existing Nodes and Functionality	104

6.30	Solution #30: Improve correctness of NWDAF analytics	104

6.30.1	Description	104

6.30.2	Procedures	105

6.30.2.1	Procedures for improving model performance based on analytics correctness evaluation of one candidate model	105

6.30.2.2	Procedures for improving model performance based on analytics correctness evaluation of multiple candidate models	106

6.30.3	Impacts on existing nodes and functionality	107

6.31	Solution #31: Multiple Analytics outputs based NF action decision	107

6.31.1	Description	107

6.31.2	Procedures	107

6.31.3	Impacts on existing nodes and functionality	108

6.32	Solution #32: Enhanced ML model provisioning	108

6.32.1	Description	108

6.32.2	Procedures	109

6.32.3	Impacts on Existing Nodes and Functionality	111

6.33	Solution #33: Improving correctness of NWDAF analytics by providing correction information	111

6.33.1	Description	111

6.33.2	Procedures	112

6.33.3	Impacts on Existing Nodes and Functionality	113

6.34	Solution #34: Enhancing the accuracy of NWDAF Analytics	114

6.34.1	Description	114

6.34.2	Procedures	114

6.34.3	Impacts on services, entities and interfaces	115

6.35	Solution #35: Improve model training and provisioning exploiting sub-areas with similar statistical properties	116

6.35.1	Description	116

6.35.2	Procedures	118

6.35.2.1	Detecting and managing sub-areas using the Area Monitoring analytics service	118

6.35.2.2	Usage of sub-areas to improve ML model provisioning	120

6.35.3	Impacts on services, entities and interfaces	121

6.36	Solution #36: Enhanced provisioning of ML model based on information about how inference will be executed	121

6.36.1	Description	121

6.36.2	Procedures	123

6.36.3	Impacts on services, entities, and interfaces	124

6.37	Solution #37: Analytics Exchange in Home routed roaming case	124

6.37.1	Description	124

6.37.2	Procedures	125

6.37.2.1	Procedure for the service experience analytics subscription from H-NWDAF to V-NWDAF	125

6.37.3	Impacts on Existing Nodes and Functionality	127

6.38	Solution #38: Interactions between VPLMN and HPLMN for restricted data collection and analytics retrieval	127

6.38.1	Description	127

6.38.1.1	General	127

6.38.1.2	HPLMN collecting data from VPLMN	127

6.38.1.3	VPLMN consuming analytics generated by HPLMN	128

6.38.2	Procedures	129

6.38.2.1	HPLMN collecting data from VPLMN	129

6.38.2.2	VPLMN consuming analytics generated by HPLMN	130

6.38.3	Impacts on Existing Nodes and Functionality	131

6.39	Solution #39: Architecture enhancements to support Data and analytics exchange in roaming case	131

6.39.1	Description	131

6.39.1.1	General	131

6.39.2	Procedure	133

6.39.2.1	Procedure for NWDAF in HPLMN to collect data from VPLMN	133

6.39.2.2	Procedure for NWDAF consumer in HPLMN to obtain analytics from VPLMN NWDAF	135

6.39.3	Impacts on existing nodes and functionality	136

6.40	Solution #40: Data and analytics exchange for roaming UEs	136

6.40.1	Description	136

6.40.2	Procedures	136

6.40.3	Impacts on Existing Nodes and Functionality	139

6.41	Solution #41: Sending data that is about to be purged	139

6.41.1	Description	139

6.41.2	Procedures	139

6.41.3	Impacts on services, entities and interfaces	140

6.42	Solution #42: Storage and retrieval of trained ML models to/from ADRF	140

6.42.1	Description	140

6.42.2	Procedures	140

6.42.2.1	Procedure for trained ML model(s) storage in ADRF	140

6.42.2.2	Procedure for trained ML model(s) retrieval from ADRF	142

6.43	Solution #43: ML model storage in ADRF and ML model provision from ADRF	143

6.43.1	Description	143

6.43.2	Procedures	144

6.43.3	Impacts on Existing Nodes and Functionality	145

6.44	Solution #44: DCCF Reselection when multiple instances of DCCF exist in a network	145

6.44.1	Description	145

6.44.2	Procedures	146

6.44.3	Impacts on Services, Entities, and Interfaces	146

6.45	Solution #45: Managing Impact of Muting on NF Producer	146

6.45.1	Description	146

6.45.2	Procedures	147

6.45.3	Impacts on services, entities and interfaces	148

6.46	Solution #46: ADRF / NWDAF Data Storage Management	148

6.46.1	Description	148

6.46.2	Procedures	149

6.46.3	Impacts on services, entities and interfaces	151

6.47	Solution #47: Sharing models between NWDAFs	151

6.47.1	Description	151

6.47.2	Procedures	151

6.47.3	Impacts on services, entities and interfaces	152

6.48	Solution #48: PCF/UE re-evaluates the URSP rules according to NWDAF's analytics	153

6.48.1	Impacts of NWDAF analytics towards URSP rules	153

6.48.2	PCF updates or generates URSP rules according to NWDAF’s analytics	153

6.48.3	Extensions of Analytic ID = "Service Experience"	154

6.48.4	Newly introduce Analytic ID = “URSP rules experience”	157

6.48.5	Procedures for URSP rules re-evaluation in PCF and UE	158

6.48.6	Impacts on services, entities and interfaces	160

6.49	Solution #49: NWDAF assisted URSP decision for redundant transmission	160

6.49.1	Description	160

6.49.2	Procedures	161

6.49.3	Impacts on Existing Nodes and Functionality	163

6.50	Solution #50: Enhancements on QoS Sustainability analytics	163

6.50.1	Description	163

6.50.1.1	Input data	164

6.50.2	Procedures	164

6.50.3	Impacts on Existing Nodes and Functionality	165

6.51	Solution #51: Selection, Monitoring, and Maintenance of NWDAF(s) for Federated Learning in 5GC	165

6.51.1	Description	165

6.51.2	Procedures	166

6.51.2.1	Client NWDAF(s) Selection in Federated Learning Preparation Phase	166

6.51.2.2	NWDAFs Monitoring and Re-selection in Federated Learning Execution Phase	167

6.51.2.3	Dynamic Discovery and Joining of New NWDAF(s) in Federated Learning Execution Phase	168

6.51.2.3.1	New Client NWDAF(s) Inform Server NWDAF Directly	168

6.51.2.3.2	Server NWDAF Gets the Information of New Client NWDAF(s) via NRF	169

6.51.3	Impacts on services, entities and interfaces	169

6.52	Solution #52: FL training update to NWDAF containing AnLF from NWDAF containing MTLF	170

6.52.1	Description	170

6.52.2	Procedures	170

6.53	Solution #53: Horizontal Federated Learning with Multiple NWDAF	171

6.53.1	Description	171

6.53.2	Procedures	172

6.53.2.1	Procedure for registration multiple NWDAF (containing MTLF) with capability of Horizontal Federated Learning	172

6.53.2.2	Horizontal Federated Learning Training triggered by MTLF with FL Server capability	173

6.53.2.3	Horizontal Federated Learning Training triggered by MTLF only with FL Client capability	174

6.53.2.4	Horizontal Federated Learning training procedure	175

6.53.3	Impacts on existing nodes and functionality	175

6.54	Solution #54: Finer granular location information based on LCS input data	176

6.54.1	Description	176

6.54.2	Procedures	177

6.54.3	Impacts on Existing Nodes and Functionality	177

6.55	Solution #55: location information with finer granularity in horizontal and vertical directions	177

6.55.1	Description	177

6.55.2	Input Data	177

6.55.3	Output Data	178

6.55.4	Procedures	179

6.55.5	Impacts on Existing Nodes and Functionality	179

6.56	Solution #56: PSAP resolution with finer granularity of location information	180

6.56.1	Description	180

6.56.2	Functional description	180

6.56.3	Procedures	180

6.56.4	Impacts on services, entities and interfaces	181

6.57	Solution #57: NWDAF determines granularity when the consumer requests finer granularity location information	181

6.57.1	Description	181

6.57.2	Input Data	182

6.57.3	Output Analytics	182

6.57.4	Procedures	183

6.57.5	Impacts on Existing Nodes and Functionality	184

6.58	Solution #58: Supporting UE mobility analytics with finer granularity than TA/cell	184

6.58.1	Description	184

6.58.2	Procedures	184

6.58.3	Impacts on services, entities and interfaces	186

6.59	Solution #59: Enhancement of NWDAF with location accuracy prediction	187

6.59.1	Functional Description	187

6.59.1.1	General	187

6.59.2	Procedure	188

6.59.3	Impacts on services, entities and interfaces	189

6.60	Solution #60: Interactions with MDAF	189

6.60.1	Description	189

6.60.1.1	Input data	189

6.60.2	Procedures	190

6.60.3	Impacts on Existing Nodes and Functionality	190

7	Overall Evaluation	190

8	Conclusions	191

Annex A: Change history	192

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, certain modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

NOTE 1:	The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

NOTE 2:	The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

NOTE 3:	The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

NOTE 4:	The constructions "can" and "cannot" shall not to be used as substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

NOTE 5:	The constructions "is" and "is not" do not indicate requirements.

1	Scope

This study focuses on key issue description, solution and evaluation/conclusion on further enhancement for network automation, as documented in TS 23.288 [5], by covering the following objectives:

-	Study possible mechanisms for improved correctness of NWDAF analytics;

-	Investigate whether and how NWDAF can assist application detection;

-	Investigate whether and how to support data and analytics exchange in roaming case;

-	Investigate data collection and data storage enhancements (including DCCF and ADRF enhancements, e.g. DCCF relocation, ADRF selection, ML model storage);

-	Study whether and how to enhance trained ML Model sharing for different vendors;

-	Study whether and how interactions between NWDAF can leverage MDAS/MDAF functionality for data collection and analytics;

-	Investigate NWDAF-assisted URSP;

-	Study whether and how to enhance the QoS sustainability analytics with finer granularity and additional input data;

-	Study whether and how to enhance architecture to support federated learning in the 5GC; and

-	Study NWDAF enhancements considering the finer granularity of location information than TA and cell level.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[6]	3GPP TS 22.071: "Location Services (LCS); Service description; Stage 1".

[7]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[8]	ITU-T Y.1540: "Internet protocol data communication service - IP packet transfer and availability performance parameters".

[9]	3GPP TS 28.554: "5G end to end Key Performance Indicators (KPI)".

[10]	3GPP TS 23.003: "Numbering, addressing and identification".

[11]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[12]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[13]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS)".

[14]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[15]	3GPP TS 28.104:"Management and orchestration; Management Data Analytics (MDA)".

3	Definitions of terms and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1], TS 23.501 [2] and TS 23.503 [4] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1], TS 23.501 [2] and TS 23.503 [4].

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], TS 23.501 [2] and TS 23.503 [4] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1], TS 23.501 [2] and TS 23.503 [4].

4	Architectural Assumptions and Principles

The architecture for the present study shall be based on the existing NWDAF framework as specified in TS 23.288 [5], TS 23.501 [2], TS 23.502 [3] and TS 23.503 [4].

Solutions shall comply with the 5G System architectural principles in TS 23.501 [2], and network data analytics principles in TS 23.288 [5].

In addition, the following architectural assumptions are followed:

-	In terms of user data privacy and security improvement, the cooperation with SA WG3 is needed;

-	For how to collect user plane data, the cooperation with FS_UPEAS is needed.

5	Key Issues

5.1	Key Issue #1: How to improve correctness of NWDAF analytics

5.1.1	Description

Correctness of predictions is usually associated to accuracy, which represents the most prominent KPI to rate ML models. However, the accuracy can be corrupted by a wrong calculation and/or unfair feedback collection. It is thus of utmost importance to ensure that the accuracy is meaningful, consistent, and comparable.

Incorrect predictions can be due to the fact that the accuracy of an ML model during inference may be lower than the accuracy of the same ML model during training. This is likely to happen if the training data set differs significantly in terms of distribution and features from the input data that the ML model is fed with during inference. Knowing how the ML model is meant to be used would help to train the ML model efficiently and effectively.

The following aspects need to be studied:

-	How to detect that improving the correctness of an Analytics ID is needed?

-	How and what information is required to compute and represent the correctness of NWDAF analytics in a reliable way?

NOTE:	Whether the information is used to represent correctness of NWDAF analytics may depend on the use case.

-	Whether and what action(s) should be taken when the NWDAF Analytics are considered incorrect, whether and how a NF can continue consuming an Analytics ID for which the need for improvement has been detected?

-	Whether and which action(s) should be taken by consumer NF to help improve correctness of NWDAF Analytics.

-	Whether and how the NWDAF can improve correctness of NWDAF Analytics (e.g. which existing and/or additional information may be used by the NWDAF to improve the correctness of NWDAF Analytics)?

-	Whether there is a need for architectural and/or functional enhancements for improving correctness of NWDAF Analytics, if there is additional signalling load caused by any new functional enhancement and how to mitigate the signalling?

-	How and which information is needed to enhance the ML model provisioning to improve the correctness of NWDAF Analytics.

-	Study mechanisms to detect that degradation on an ML model has happened and whether and which actions should be triggered.

5.2	Key Issue #2: NWDAF-assisted application detection

5.2.1	Description

This key issue proposes to study whether and how the NWDAF can assist the detection of the traffic generated by an application. This KI corresponds to WT#2.1: NWDAF assisted application detection.

The detection of traffic generated by an application can be performed using the application detection filters in the UPF/SMF and those application detection filters may include Packet Flow Description(s), i.e. PFD(s). Depending on service level agreements between the operator and the Application Service Provider (ASP), the ASP may provide PFD(s) for each application identifier maintained by the ASP.

It will be studied whether and how the NWDAF can assist to identify the traffic specific to a certain application at the UPF.

The following issues shall be studied:

-	How to consider the consent of user and ASP (i.e. no privacy and regulatory issue) for performing and exposing the analytics for application detection?

-	Whether and how the NWDAF can assist the application detection, considering the following aspects:

-	Study use cases where the NWDAF can assist the application detection, for instance how to detect the application traffic if the ASP provides initial PFD information but does not update it in time or does not update it anymore, or if the ASP does not provide any PFD information.

-	If NWDAF provides analytics to assist the application detection performed at the UPF. What is the potential consumer of these analytics (e.g. NEF/PFDF) and how does that consumer use these analytics?

-	Whether new input data needs to be collected by NWDAF to assist the application detection performed at the UPF?

-	Whether existing or new Analytics ID(s) are needed to be provided by NWDAF to assist the application detection performed at the UPF?

Solutions for this key issue shall not cause degradation of UPF performance.

NOTE:	Coordination with FS_ UPEAS / Study on UPF enhancement for Exposure and SBA may be needed.

5.3	Key Issue #3: Data and analytics exchange in roaming case

5.3.1	Description

In roaming scenario, the HPLMN/VPLMN may need to collect data or consume analytics from the VPLMN/HPLMN. The data or analytics may relate to particular UEs or contain information about all UEs or groups of UEs. Both PLMNs (VPLMN, HPLMN) need the ability to control the amount of data exposed and to abstract or hide network-internal aspects based on user consent, operator policy, regulatory constraints and/or roaming agreements. Use cases for data exchange and analytics exposure between HPLMN and VPLMN require further investigation.

The technical aspects to be studied include the following:

-	Identification of use cases and requirements for data and/or analytics exchange between PLMNs (i.e. HPLMN and VPLMN(s)) in roaming scenario. For each use case, identify what raw data, existing or new Analytics IDs needs to be exchanged, and which existing Analytics ID can be enhanced or new Analytics ID can be generated for the roaming user based on the exchanged data

-	Determination of possible architecture enhancements to support this exchange in roaming scenarios and of any necessary enhancements to related NFs in HPLMN and VPLMN, e.g. possible enhancement on data collection using DCCF, or possible enhancement on data storage using ADRF, or possible enhancement on security and privacy of the data and analytics exchange between PLMNs.

NOTE 1:	Where possible, existing capabilities of the 5GC for inter-PLMN communication should be reused, for instance NRF capabilities to authorize access to services, and capabilities of Security Edge Protection Proxy (SEPP) defined in TS 33.501 [12] to manipulate data and to secure the confidentiality, integrity and authenticity of data exchange over the N32 interface between PLMNs.

NOTE 2:	Coordination with SA3 on security aspects is required.

NOTE 3:	Coordination with GSMA on sharing of user data while roaming may be required.

5.4	Key Issue #4: How to Enhance Data collection and Storage

5.4.1	Description

For this Key Issue the following aspects will be studied:

-	Interaction between multiple DCCFs and MFAFs (e.g. DCCF or MFAF relocation) if multiple DCCFs are deployed in one PLMN, to facilitate and improve data collection coordination.

-	Whether and how to enhance ADRF selection in case of multiple ADRF deployed in the network and whether and how to support ADRF relocation.

-	Whether and how the ADRF should store types of data other than historical data and analytics (e.g. ML models, analytics context) for network analytics.

-	Whether and what other enhancements are required for storage of data and/or analytics in ADRF, NWDAF and/or data source NF.

-	Whether and what other enhancements can be made to further reduce signalling and data traffic and the impact of obtaining data on data sources related to network analytics.

5.5	Key Issue #5: Enhance trained ML Model sharing

5.5.1	Description

As described in the below note from clause 5.1, TS 23.288 [5] that in Rel-17 the ID(s) of the NWDAF containing MTLF(s) is locally configured in the NWDAF containing AnLF, and the NWDAF containing AnLF is only allowed to retrieve trained ML model(s) from the configured NWDAF containing MTLF(s).

NOTE 3:	In this Release of the specification an NWDAF containing AnLF is locally configured with (a set of) IDs of NWDAFs containing MTLF and the Analytics ID(s) supported by each NWDAF containing MTLF to retrieve trained ML models. An NWDAF containing AnLF uses NWDAF discovery for NWDAF containing MTLF within the set of configured IDs of NWDAFs containing MTLF, if necessary. ML Model provisioning/sharing between multiple MTLFs is not supported in this Release of the specification.

In Rel-18, the following aspects are to be studied to enhance trained ML Model sharing between NWDAFs from different vendors:

-	How to discover and select a NWDAF containing MTLF even from different vendors which can provide interoperable trained ML model(s), and how to retrieve interoperable trained ML model(s) from it, by a NWDAF containing AnLF, including:

-	Whether and what new functionality / function(s) is needed for support of trained ML model interoperability?

-	What additional information should be exchanged between the NWDAF containing MTLF and the NWDAF containing AnLF, for support of trained ML model interoperability.

-	Whether and how to define common characteristics for trained ML models from different vendors?

NOTE 1:	Coordination with SA5 is needed for ML model sharing aspects.

NOTE 2:	Coordination with SA3 is needed for security aspects related to ML model sharing (e.g. ML model integrity, authorization of use, etc.) as investigated by this Key Issue.

5.6	Key issue #6: NWDAF-assisted URSP

5.6.1	Description

NWDAF can already provide some network analytics to PCF for policy decisions as described in clause 6.1.1.3 in TS 23.503 [4]. However, further investigation is required on whether and how analytics can be used to assist in the generation of URSP Rules.

In this key issue, the following aspects need to be studied:

-	Whether any and which components of the URSP rules can benefit from analytics.

-	Whether and how existing Analytics IDs, or new Analytics ID(s) can be used to assist in the generation of URSP Rules.

-	What procedures trigger the subscription to these Analytics IDs.

-	Whether new (set of) interactions(s) are required to assist in the generation of URSP Rules as defined in Rel17, and how to define the new interactions if needed.

-	What information should be collected (or provided) as input (or output) by the NWDAF for these Analytics IDs.

5.7	Key Issue #7: Enhancements on QoS Sustainability analytics

5.7.1	Description

Currently, NWDAF can provide Cell level QoS Sustainability analytics with limited QoS parameters, i.e. RAN UE Throughput and QoS Flow retainability. More KPIs reporting for QoS Sustainability analytics would benefit, e.g. for V2X use cases as defined by 5GAA.

Therefore, the following aspects are needed to be studied for QoS Sustainability analytics enhancement:

-	Whether and how to enhance the QoS Sustainability analytics with a finer granularity area (e.g. below Cell level)? Or alternatively if a new Analytics ID is required for the finer granularity area for this purpose?

-	Whether and how to enhance the QoS Sustainability analytics (or a new Analytics ID) with additional input and/or output data?

-	Whether and how to enhance the request for QoS Sustainability analytics (or a new Analytics ID)?

NOTE:	Coordinated activities with 5GAA are needed.

5.8	Key Issue #8: Supporting Federated Learning in 5GC

5.8.1	Description

This contribution is related to WT # 4.1.

Current enablers for network automation architecture by NWDAF still faces some major challenges as follows:

-	User data privacy and security (protected by e.g. GDPR) has become a worldwide issue, it is also difficult for NWDAF to collect UE level network data.

-	With the introduction of MTLF in Rel-17, various data from wide area is needed to train an ML model for NWDAF containing MTLF. However, it is difficult for NWDAF containing MTLF to collect all the raw data from distributed data source in different areas.

In order to address the challenges, 3GPP tries to adopt Federated Learning (also called Federated Machine Learning) technique in NWDAF containing MTLF to train an ML model, in which there is no need for raw data transferring (e.g. centralized into NWDAF) but only need for cooperation among multiple NWDAFs (MTLF) i.e. sharing of ML model and of the learning results among multiple NWDAFs (MTLF). In Rel-17, however, the cooperation of multiple NWDAF containing MTLF is explicitly prohibited and it is only allowed for NWDAF containing AnLF to subscribe or request the ML model from the configured NWDAF containing MTLF

This Key Issue is aim to study architecture enhancement to support Federated Learning which allows the cooperation of multiple NWDAF containing MTLF to train an ML model in 3GPP network with the following aspects:

-	Identify the use cases that required Federated learning in 5GC;

-	Study the registration and discovery of the NWDAF supporting Federated Learning;

-	Study how to decide whether Federated Learning is required or not for an existing Analytics ID or a new Analytics ID;

-	Study how to coordinate multiple NWDAFs including selection of participant NWDAF instances in the Federated Learning group, e.g. assistance information (if any) to perform the selection, and decision of role for the participant NWDAF;

-	Study whether and how to perform performance (e.g. network performance and model performance) monitoring of the NWDAF Federated Learning operation.

NOTE 1:	Performance monitoring of Federated Learning operation should be aligned with mechanisms for improved correctness of analytics defined in WT#1.2.

NOTE 2:	In terms of user data privacy and security improvement, the cooperation with SA3 is needed.

NOTE 3:	The impact on UE and RAN shall be avoided for this Key Issue.

NOTE 4:	Solutions requiring model distribution for FL should be aligned with mechanism for model sharing defined in WT#3.2.

NOTE 5:	Server NWDAF connects to one layer of Client NWDAFs, and any of the Client NWDAFs cannot cascade more sublayers.

NOTE 6:	All the NWDAFs attending the Federated Learning should belong to the same PLMN.

5.9	Key Issue #9: Enhancement of NWDAF with finer granularity of location information

5.9.1	Description

The NWDAF can retrieve and collect UE location information and then provide some analytic(s) to NWDAF consumer(s), e.g. UE mobility analytics, QoS Sustainability Analytics as defined in TS 23.288 [5]. However, the UE location information that NWDAF can obtain is only TA/cell granularity in Rel-17.

The horizontal accuracy and the vertical accuracy of the existing location service can reach a granularity level finer than TA and cell level. Meanwhile, some extra information (e.g. speed, heading) could possibly also be provided by the location service. But whether such LCS related information is beneficial for the NWDAF is not clear, and it also needs to be studied how the NWDAF can obtain such LCS related information. Therefore, a KI to study whether and how the NWDAF can provide additional benefits from location service is required.

In this key issue, the following aspects will be studied:

-	Identify use case(s) and corresponding existing or new Analytics ID(s) where the analytics require location information with finer granularity than TA/cell level, and how to enhance related existing Analytics ID(s).

-	Identify how an NWDAF determines that location information with finer granularity than cell/TA level is required in output analytics.

-	Identify what input data needs to be collected to deliver analytics with fine granularity location information.

-	Identify how NWDAF acquires the input data to deliver finer granularity location information. Whether and how the functionality and services of NWDAF or other NF(s) need to be enhanced.

NOTE 1:	Coordinated activities with the eLCS_ph3 study are needed.

NOTE 2:	Some examples of UE location different than cell/TA level are described in clause 4.2 and 4.3, TS 22.071 [6].

5.10	Key Issue #10: Interactions with MDAS/MDAF

5.10.1	Description

MDAS/MDAF functionality enables a service consumer to obtain management data analytics, and NWDAF can be one of such service consumers. The following aspects shall be studied:

-	Identify which use case(s) should be applied for the interactions between NWDAF and MDAS/MDAF.

-	Whether and how NWDAF can leverage MDAS/MDAF functionality, for producing Analytics ID(s) given in TS 23.288 [5].

-	Whether and how NWDAF can leverage MDAS/MDAF functionality for data collection.

-	How MDAS/MDAF instances are discovered and selected by NWDAF.

NOTE 1:	Solution impacts to MDAS/MDAF need SA5 to give inputs before conclusions are made.

NOTE 2:	Studying the MDAS/MDAF output is out scope of this KI.

6	Solutions

6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of solutions to key issues

|  | Key Issues |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| 1 | X |  |  |  |  |  |  |  |  |  |
| 2 | X |  |  |  |  |  |  |  |  |  |
| 3 | X |  |  |  |  |  |  |  |  |  |
| 4 | X |  |  |  |  |  |  |  |  |  |
| 5 | X |  |  |  |  |  |  |  |  |  |
| 6 | X |  |  |  |  |  |  |  |  |  |
| 7 | X |  |  |  |  |  |  |  |  |  |
| 8 |  | X |  |  |  |  |  |  |  |  |
| 9 |  | X |  |  |  |  |  |  |  |  |
| 10 |  |  | X |  |  |  |  |  |  |  |
| 11 |  |  | X |  |  |  |  |  |  |  |
| 12 |  |  |  | X |  |  |  |  |  |  |
| 13 |  |  |  |  | X |  |  |  |  |  |
| 14 |  |  |  |  | X |  |  |  |  |  |
| 15 |  |  |  |  | X |  |  |  |  |  |
| 16 |  |  |  |  |  | X |  |  |  |  |
| 17 |  |  | X |  |  | X |  |  |  |  |
| 18 |  |  |  |  |  |  | X |  |  |  |
| 19 |  |  |  |  |  |  | X |  |  |  |
| 20 |  |  |  |  |  |  | X |  |  |  |
| 21 |  |  |  |  |  |  |  | X |  |  |
| 22 |  |  |  |  |  |  |  | X |  |  |
| 23 |  |  |  |  |  |  |  | X |  |  |
| 24 |  |  |  |  |  |  |  | X |  |  |
| 25 |  |  |  |  |  |  |  |  | X |  |
| 26 |  |  |  |  |  |  |  |  | X |  |
| 27 |  |  |  |  |  |  |  |  | X |  |
| 28 | X |  |  |  |  |  |  |  |  |  |
| 29 | X |  |  |  |  |  |  |  |  |  |
| 30 | X |  |  |  |  |  |  |  |  |  |
| 31 | X |  |  |  |  |  |  |  |  |  |
| 32 | X |  |  |  |  |  |  |  |  |  |
| 33 | X |  |  |  |  |  |  |  |  |  |
| 34 | X |  |  |  |  |  |  |  |  |  |
| 35 | X |  |  |  |  |  |  |  |  |  |
| 36 | X |  |  |  |  |  |  |  |  |  |
| 37 |  |  | X |  |  |  |  |  |  |  |
| 38 |  |  | X |  |  |  |  |  |  |  |
| 39 |  |  | X |  |  |  |  |  |  |  |
| 40 |  |  | X |  |  |  |  |  |  |  |
| 41 |  |  |  | X |  |  |  |  |  |  |
| 42 |  |  |  | X |  |  |  |  |  |  |
| 43 |  |  |  | X | X |  |  |  |  |  |
| 44 |  |  |  | X |  |  |  |  |  |  |
| 45 |  |  |  | X |  |  |  |  |  |  |
| 46 |  |  |  | X |  |  |  |  |  |  |
| 47 |  |  |  |  | X |  |  |  |  |  |
| 48 |  |  |  |  |  | X |  |  |  |  |
| 49 |  |  |  |  |  | X |  |  |  |  |
| 50 |  |  |  |  |  |  | X |  |  |  |
| 51 |  |  |  |  |  |  |  | X |  |  |
| 52 |  |  |  |  |  |  |  | X |  |  |
| 53 |  |  |  |  |  |  |  | X |  |  |
| 54 |  |  |  |  |  |  |  |  | X |  |
| 55 |  |  |  |  |  |  |  |  | X |  |
| 56 |  |  |  |  |  |  |  |  | X |  |
| 57 |  |  |  |  |  |  |  |  | X |  |
| 58 |  |  |  |  |  |  |  |  | X |  |
| 59 |  |  |  |  |  |  |  |  | X |  |
| 60 |  |  |  |  |  |  |  |  |  | X |

  - 6.1	Solution #1: Improving Correctness using multiple ML models for an analytics report

6.1.1	Description

TS 23.288 [5] assumes that the NWDAF containing AnLF has a single ML model for inferring analytics, at a given time, after applying any potential analytics filter. However, in many scenarios, it is possible that the NWDAF containing AnLF may have multiple ML models at its disposal, and therefore, multiple ML models can be used to generate the analytic reports provided by a given Analytics ID.

Having the NWDAF containing AnLF multiple ML models to choose from, enables use cases where an ML model can be selected depending on, e.g. the UE location, time of day, network load, or any other filter of information.

This solution is based on the existence of several models to draw a prediction for a single Analytics ID. In all these scenarios, the following functionalities needs to be taken into consideration:

-	Each ML model needs to have a unique identifier within the PLMN which is assigned by the ML model creator, i.e. an NWDAF containing MTLF. The ML Model unique identifier may be created, e.g. upon the NF ID of the NWDAF containing an MTLF and a serial number.

Editor's note:	Who and how to create the unique identifier for ML model is FFS.

-	An NWDAF containing AnLF should be able to get all available ML models for a specific Analytics ID, from an NWDAF containing MTLF.

-	For the case of ML model provisioning, an NWDAF containing AnLF should be able to request ML model information (i.e. the parameters described in clause 6.2A.2 of TS 23.288 [5]) for an Analytics ID, from an NWDAF containing MTLF, and be able to receive responses including more than one ML model for the same Analytics ID.

-	Upon reception of multiple ML model information for the same Analytics ID, the NWDAF containing AnLF should be able to choose and download the model(s) of its choice.

Figure 6.1.1-1 presents an example of an NWDAF containing AnLF that collects multiple data, feeds each data to a multiple and different ML models, and uses three ML models for generating predictions for the same Analytics ID. Once a given ML model has generated a prediction, an internal logic in the aggregation/voting unit selects the best prediction and delivers it to the consumer. Therefore, the consumer is provided with the best of the three predictions or an optimized aggregated prediction.

Figure 6.1.1-1: Aggregation/voting chooses the best prediction

NOTE:	The aggregation/voting function in Figure 6.1.1-1 is for illustration purposes and is left to implementation logic.

6.1.2	Procedures

6.1.2.1	General

This solution does not require new procedures. However, the existing procedures for provisioning an ML model are extended to allow the description of multiple ML models: the data of several service operations are modified to accommodate the reference to multiple ML models and the delivery of multiple predictions.

6.1.2.2	Support for describing multiple ML models in provisioning service operations

After the NWDAF containing AnLF selects the proper NWDAF containing MTLF via the NRF procedures, figure 6.1.2.2-1 illustrates a subscription using the Nnwdaf_MLModelProvision_Subscribe service operation for a given Analytics ID. This service operation is enhanced with an indication from the consumer declaring support for multiple ML models.

Then an Nnwdaf_MLModelProvision_Notify request is enhanced with the capability of including, for a given Analytics ID, pairs of unique identifiers of the ML model and its corresponding ML model information (as per clause 6.2A.2 of TS 23.288 [5]).

Then, the NWDAF containing AnLF can find out whether it has already downloaded any of the ML models identified by the unique identifier and may decide to download one or more ML models of its choice which has not already downloaded.

The NWDAF containing MTLF may provide all pairs of unique identifiers of the ML model and its corresponding ML model information to the NWDAF containing MTLF ANLF.

Editor's note:	How to resolve ML Model data privacy and improve the efficiency is FFS in case of the NWDAF containing MTLF providing all all pairs of unique identifiers of the ML model and its corresponding ML model information.

Figure 6.1.2.2-1: Enhancements to Nnwdaf_MLModelProvision services

Figure 6.1.2.2-2 illustrates an NWDAF containing AnLF sending an Nnwdaf_MLModelInfo_Request request for a given Analytics ID to an NWDAF containing MTLF. This service operation is enhanced with an indication from the consumer declaring support for multiple ML models.

The NWDAF containing MTLF sends an enhanced response including, for a given Analytics ID, pairs of unique identifiers of the ML model and its corresponding ML model information (the ML model information is described in clause 6.2A.2 of TS 23.288 [5]).

Figure 6.1.2.2-2: Enhancements to Nnwdaf_MLModelInfo service

6.1.3	Impacts on existing nodes and functionality

The interfaces related to ML Model provisioning are updated to support multiple ML model in the following ways:

-	Nnwdaf_MLModelProvision_Subscribe and Nnwdaf_MLModelInfo_Request are enhanced with an indication, inserted by the consumer, indicating whether the consumer supports multiple ML models for the same Analytics ID.

-	Nnwdaf_MLModelProvision_Notify and the response to Nnwdaf_MLModelInfo_Request are enhanced for supporting the provisioning of data related to multiple ML models for the same Analytics ID, including pairs of unique identifiers of the ML Model together with its corresponding ML Model information.

6.2	Solution #2: Improving the Correctness of Service Experience Predictions with Contribution Weights

6.2.1	Description

This solution addresses the following aspects of Key Issue #1 for the Service Experience Analytics ID.

-	How and what information is required to compute and represent the correctness of NWDAF analytics in a reliable way?

-	Whether and how the NWDAF can improve correctness of NWDAF Analytics (e.g. which existing and/or additional information may be used by the NWDAF to improve the correctness of NWDAF Analytics)?

When Service Experience information is provided to the NWDAF for multiple UEs, the NWDAF does not know the relative importance of the Service Experience value (i.e. MOS) that was provided by each UE. For example, it might be that one the Service Experience of one UE is not very important because the UE is an infrequent user of the service, or the UE does not use all features that are associated with the service. Whereas another UE might be a frequent user of the service or a "power user" who uses many features that are associated with the service.

This solution proposes that, when the AF provides Service Experience Information for a group UEs, the AF can also provide a "Service Experience Contribution Weight" with each UE's Service Experience value. The "Service Experience Contribution Weight" is determined by the AF and indicates the relative importance of each UE's Service Experience. The NWDAF can then use this information to derive more accurate predictions and confidence values. For example, the NWDAF may choose to give less weight, or importance to a Service Experience value that is associated with a "Service Experience Contribution Weight" that is lower than other "Service Experience Contribution Weights". The format of the "Service Experience Contribution Weight" may be defined by stage-3 but an example is that the "Service Experience Contribution Weight" may be an integer value with a range that is agreed on between the MNO and AF where a higher value indicates more importance than a lower value. The NWDAF may use the "Service Experience Contribution Weight" values the improve the correctness of its Service Experience predictions and confidence values calculations (e.g. giving less weight to less important values will result in improved correctness of its predictions).

6.2.2	Procedures

The NWDAF subscribes to the service data from the AF (i.e. the information in the Table 6.4.2-1 of TS 23.288 [5]) either directly for trusted AFs by invoking Naf_EventExposure_Subscribe service (Event ID = Service Experience information, Event Filter information = Area of Interest, Application ID) as defined in TS 23.502 [3], or indirectly for untrusted AFs via NEF by invoking Nnef_EventExposure_Subscribe service (Event ID = Service Experience information, Event Filter information = Area of Interest, Application ID).

When the AF provides the Service Experience Information (i.e. the information in the Table 6.4.2-1 of TS 23.288 [5]) the AF can also provide a "Service Experience Contribution Weight" for each Service Experience value.

The NWDAF considers the "Service Experience Contribution Weight" values when calculating Service Experience predictions and confidence values. How the NWDAF uses the "Service Experience Contribution Weight" values when calculating Service Experience predictions and confidence values is left to NWDAF implementation and is not in scope of 3GPP.

6.2.3	Impacts on existing nodes and functionality

AF:

-	Sends the "Service Experience Contribution Weight" when sending a notification in response to an Naf_EventExposure_Subscribe service invocation (Event ID = Service Experience information).

NWDAF:

-	Receives the "Service Experience Contribution Weight" when receiving a notification in response to an Naf_EventExposure_Subscribe service invocation (Event ID = Service Experience information).

6.3	Solution #3: Accuracy based NWDAF Analytics Correctness Improvement

6.3.1	Description

This solution is about KI #1: How to improve correctness of NWDAF analytics to address how to improve correctness of NWDAF analytics.

In AI community, there is a concept called 'accuracy' which is used to indicate the correctness of an AI/ML model's predictions. The Accuracy is the percentage of correct predictions in all predictions. One of methods to determines a prediction is correct is to compare the prediction with the label (data). A set of data can be composed as (x, t), and the 'x' is 'input data' and 't' is 'label' or 'label data'. If this label is correct, it also is called 'ground truth'.

For the regression problems, the predictions and label data may be not same but similar, and their difference can be calculated by other methods e.g. Mean Absolute Error (MAE). If this difference is smaller than a threshold, this prediction should be considered as correct.

Figure 6.3.1-1: Accuracy in AI Community

As shown in Figure 6.3.1-2, Accuracy in Training (AiT) and Accuracy in Use (AiU) can be used to measure the correctness of a ML model, the Accuracy in Use should be observed/monitored by the NWDAF/network, which could be a trigger for NWDAF(MTLF) to re-train the ML Model if Accuracy in Use cross a threshold:

Figure 6.3.1-2: Accuracy in Training with Training dataset vs Accuracy in Use with live network dataset

As shown in left side of the Figure, a NWDAF(MTLF) trained a ML Model with Training dataset (input data, label data) and the Accuracy in Training is to indicate the performance of ML model in training stage by comparing prediction with label data in validation dataset reserved from training dataset.

The NWDAF(MTLF) deliver the trained ML Model to a NWDAF(AnLF), which will be used by the NWDAF(AnLF) to perform inference with input data from live network.

3)	As shown in right side of the Figure, Accuracy in Use is to indicate the performance of ML model used in live network by comparing prediction with the observed label data i.e. ground truth from the live network.

Please note that the NWDAF(AnLF) performs inference with live network input data to get the prediction and also retrieves the label data from live network.

4)	However, due to some reasons, e.g. different data distribution between training data set and data in live network, poor model generalization ability, etc. there may be a gap between Accuracy in Training and Accuracy in Use, and the Accuracy in Use usually is not as good as Accuracy in Training especially as times goes by.

The NWDAF(AnLF) decides to indicate the NWDAF(MTLF) to re-train the AI/ML model e.g. if Accuracy in Use crosses a threshold or the gap between Accuracy in Training and Accuracy in Use crosses a threshold.

In order to improve training data quality e.g. the data distribution, the NWDAF(AnLF) also provides the input data, labelled data, where prediction is regarded as incorrect prediction by comparing with labelled data i.e. ground truth, to help the NWDAF(MTLF) to re-train a better AI/ML Model e.g. with model generalization ability.

Similarly, regarding regression, MAE in training and MAE in use is introduced to measure the ML model correctness.

6.3.2	Procedures

Figure 6.3.2-1 depicts the procedure for the proposed solution that the AnLF collects the label data and calculates the AiU.

Figure 6.3.2-1: AnLF collect label data and calculate accuracy in use

1.	The NWDAF(AnLF) service consumer subscribes to analytics information by invoking the Nnwdaf_AnalyticsSubscription_Subscribe service operation. The parameters that can be provided by the NWDAF service consumer are listed in clause 6.1.3 of TS 23.288 [5].

2.	The NWDAF(AnLF) subscribes to a NWDAF(MTLF) a trained ML Model by invoking the Nnwdaf_MLModelProvision (an Analytics ID) service operation.

3~4.	The MTLF trains the model according to the requirement from AnLF and notifies the AnLF with the ML model information by invoking Nnwdaf_MLModelProvision_Notify (Analytic ID, the file address of the trained ML model, Accuracy in Training (AiT) or MAE in Training) service operation.

5~8.	AnLF collects the input data from input data providers, e.g. OAM, NFs, makes predictions and sends the predictions to the NWDAF consumer NF.

9~10.	the AnLF collects label data corresponding to predictions from data providers e.g. AF and SMF, and calculates the Accuracy in Use or the MAE in Use, which reflects the performance of ML model used in live network by comparing prediction with the observed labelled data i.e. ground truth from the live network.

11.	The AnLF compares the Accuracy in Use with the Accuracy in Training and/or MAE in Use with MAE in Training and decide whether or not to send notification to MTLF to re-train the AI/ML model e.g. if Accuracy in Use cross a threshold or by comparing the between Accuracy in Training and Accuracy in Use.

12~13.	The AnLF reuse MLModelProvision service (subscription Correlation ID, Analytic ID, AiU or MAE in Use, input data and label data) to modify of the original ML model subscription in step 2, indicating to the MTLF that the Accuracy in Use or MAE in Use is not good as the Accuracy in Training or MAE in Training and the MTLF retrains the model with new data from the AnLF.

In order to improve training data quality e.g. the data distribution, the AnLF provides the input data, label data i.e. ground truth, where prediction is regarded as incorrect prediction by comparing with labelled data i.e. ground truth, to help the MTLF to re-train a better AI/ML Model e.g. with model generalization ability.

Editor's note:	How the NWDAF(AnLF) determines the prediction is regarded as an incorrect prediction by comparing with the ground truth is FFS.

14~15.	The AnLF notifies the consumer that the Accuracy in Use is not as good as the Accuracy in Training to consequently stop using the provided analytic result by invoking the Nnwdaf_AnalyticInfo_Notify (Analytic ID, AiU, Disable notification).

6.3.3	Impacts on services, entities and interfaces

The solution has the following impacts:

NWDAF(AnLF):

-	Collects data from corresponding data provider(s) to calculate the Accuracy in Use.

-	Sends the notifications to the NWDAF(MTLF) and consumer to inform the Accuracy in Use or MAE in Use has decreased.

NWDAF(MTLF):

-	Receives the notification that the Accuracy in Use or MAE in Use has decreased from NWDAF(AnLF) and re-trains the AI/ML Model.

NWDAF service Consumer:

-	Receives the notification that the Accuracy in Use or MAE in Use has decreased from NWDAF(AnLF) and stops using the analytics provided by the NWDAF(AnLF).

6.4	Solution #4: Determining ML model drift for improving analytics accuracy

6.4.1	Description

The accuracy of analytic output from an NWDAF depends very much on the accuracy of the ML model provided by the MTLF NWDAF.

The training data that are used to train an ML model are usually historical data (data stored in the ADRF). The validity/accuracy of the ML model depends on whether the training data used are up to date with the real-time network configuration/behaviour. For example, compared to when the training data were collected the network operator may configure additional network resources to a network slice, or the number of users accessing services via the core network may considerably increase (e.g. tourist season in the summer). Such use case may cause a model drift given that ML model was not trained with up-to-date data.

There are many reasons that ML model drift can occur but the main cause is a change of the data with time. A simple solution to this problem is to re-train an ML model periodically. Such approach will ensure that the NWDAF always uses an up-to-date training data for an ML model. However, such approach requires considerable resources and is not energy efficient. Hence a solution is required to allow the network (i.e. NWDAF) to determine when an ML model requires re-training.

The solution proposed in this paper focuses on the NWDAF to evaluate if an action taken by a consumer would result in a model drift and then evaluate if the training data are up-to-date.

A general procedure is provided in the figure below.

Figure 6.4.1-1: Model drift detected at NWDAF MTLF

The general steps followed are:

-	A consumer of analytics determines an action and optionally provides the action taken to the MTLF

-	The MTLF determines if the action taken would significantly change the behaviour of one or more NFs and determine if there could be a data drift

-	The MTLF compares the training data with real-time (or near-real time) data and evaluate if (any) data drift detected would result in an ML model drift that would reduce the accuracy of the analytics.

NOTE:	The MTLF can use statistical methods to determine if there is data drift. Such approach is out of scope of 3GPP.

6.4.2	Procedures

The procedure is shown below:

Figure 6.4.2-1: Model drift determined at MTLF NWDAF

1.	A consumer requests analytics including an Analytics ID as per TS 23.288 [5]. For example, a consumer can request analytics for an Analytics ID for NF load. The request may include a minimum accuracy may denote a confidence level of the analytics requested. The consumer may include an indication that it can provide feedback on an action taken.

2.	The AnLF request an ML model from the MTLF NWDAF to derive analytics for the Analytics ID requested if the AnLF has no ML model available for the requested Analytics ID.

3.	The MTLF NWDAF provides the current trained ML model for the Analytics ID.

4.	The MTLF provides the requested ML model to the AnLF NWDAF. Based on the feedback indication provided by the consumer the MTLF may include in the request to provide feedback when a consumer determines an action based on analytics provided by the AnLF. The feedback is used to determine whether drift detection should be carried out for an ML model (see step 10). The MTLF includes a notification correlation ID in the request.

5.	The AnLF NWDAF derives analytics.

6.	The AnLF NWDAF provides requested analytics to the consumer. The AnLF also forwards the feedback request received from the MTLF.

7.	The consumer may determine an action based on the received analytics. For example, if the analytics indicate a high load at a UPF function the consumer, i.e. SMF, may select a less loaded UPF.

8.	[CONDITIONAL] Based on the feedback indication the consumer determines to report a network behaviour change including a time where the action was taken.

NOTE:	How the consumer determines that the action taken changes the behaviour of the network is implementation specific.

9.	[CONDITIONAL] The consumer reports to the AnLF/MTLF that the action t will change the behaviour of the network.

Editor's note:	The type of actions that the consumer can provide is FFS.

10.	The MTLF determines based on the received feedback to evaluate the accuracy of the ML model. Alternatively, the MTLF at regular intervals initiates an ML model drift detection as described in steps 12-14.

11.	The MTLF NWDAF subscribes to historical data that were used to train the ML model for the specific Analytic ID from the ADRF.

12.	The MTLF NWDAF subscribes to real-time data (or near real time data) from the 5GC NFs, AF, OAM that are required to derive analytics for an Analytic ID (as described for each Analytic ID in TS 23.288 [5].

13.	The MTLF NWDAF determines model drift by comparing historical data with near real time data. The MTLF NWDAF may use statistical analysis or use AI/ML procedures to determine data/model drift (out of scope of 3GPP).

14.	If the MTLF NWDAF determines that Model drift exceed a threshold the MTLF retrains the ML model.

15.	The MTLF NWDAF may notify the AnLF that the ML model drift exceeds a threshold. The notification may notify the AnLF to stop using the ML model to provide analytics. The MTLF NWDAF may also provide an indication on when the updated ML model will be available.

16.	Based on the indication received from the MTLF the AnLF may indicate to the analytics consumer a pause in the subscription because an ML model requires retraining. The AnLF may also indicate the time new analytics will be available.

17.	The MTLF NWDAF provides the re-trained ML model to the AnLF NWDAF.

18.	The AnLF notifies the consumer to resume subscription indicating analytics are available.

6.4.3	Impacts on services, entities and interfaces

-	MTLF: Identifying ML model drift by comparing historical data with real-time data.

-	Consumer of analytics: Providing feedback to AnLF/MTLF on an action taken.

6.5	Solution #5: Enhancements on ML model provision to improve correctness of NWDAF analytics

6.5.1	Description

This is a solution for Key Issue #1: "How to improve correctness of NWDAF analytics".

This solution introduces several options for improving the correctness of NWDAF analytics by enabling the ability of self-correction on NWDAF based on closed-loop control.

Specifically, in case of analytics for prediction, the provided analytics (i.e. desired data and events for prediction) by an NWDAF can be different to actual data or events occurred in analytics target period and it may become significant faults on the consumer NF action. In order to address the potential faults on NF action, this solution proposes that the procedures for NWDAF i) to monitor the accuracy of the provided analytics, and ii) to correct potential errors on analytics as described in clause 6.5.2.

For details, NWDAF is decomposed into AnLF and MTLF which provide analytics and ML model, respectively. Since AnLF generates analytics inferred by the provisioned ML model from MTLF, the error on analytics could be incurred by inappropriate ML model provision. Thus, MTLF should monitor the error on analytics for checking accuracy of the provisioned ML model. However, the current specification does not allow that MTLF monitors the analytics generated by AnLF as well as the actual data and events during Analytics target period. To that end, this solution proposes two options: i) AnLF based error monitoring, and ii) MTLF based error monitoring as described in clauses 6.5.2.1 and 6.5.2.2, respectively.

Also, in current specification the MTLF provides ML model relying on the belief that the 5GS will be in similar state during Analytics target period as the period when the trained data had been sampled. In fact, however, the configuration updates, policy changes, and various unexpected events on 5GS would continuously occur during the analytics target period and it could incur the dynamics that the provisioned ML model have not been trained. In case, consequently, the accuracy of NWDAF analytics will be significantly degraded. In order to address the problem, this solution proposes to enable MTLF to detect the dynamics and to provide a better ML model for the system as described in clause 6.5.2.3.

6.5.2	Procedures

Figure 6.5.2-1 depicts the general procedures for this solution to enhance the accuracy of NWDAF analytics.

Figure 6.5.2-1: General procedures for enhancing the accuracy of NWDAF analytics

0.	The consumer NF subscribes to AnLF services (via DCCF) to provide a (set of) analytics.

1.	The NWDAF containing AnLF subscribes to a (set of) trained ML model(s) associated with a (set of) Analytics ID(s) by invoking Nnwdaf_MLModelProvision_Subscribe service operation.

2.	The NWDAF containing AnLF monitors error between generated analytics by the ML model and actual events, and sends the monitoring results to the NWDAF containing MTLF as described in clause 6.5.2.1.

3.	The NWDAF containing MTLF monitors difference between the data for training the ML model and the actual data as described in clause 6.5.2.2.

4.	The NWDAF containing MTLF detects the dynamics on 5GS associated with the analytics as described in clause 6.5.2.3.

5.	The NWDAF containing MTLF determines to re-select a (set of) trained ML model(s) associated with a (set of) analytics and/or to re-train the trained ML model(s) based on the retrieved results of step 2-4.

NOTE:	Each result of steps (i.e. step 2, step 3, and step 4) can trigger the decision of reselection and retraining of ML model.

6.	If the NWDAF containing MTLF select a novel trained ML model and/or re-trains the ML model, then send the ML model to the NWDAF containing AnLF by invoking Nnwdaf_MLModelProvision_Notify service operation.

6.5.2.1	Procedures for AnLF based error monitoring

Figure 6.5.2.1-1 describes the procedures that NWDAF containing AnLF monitors the error on the provided analytics from the ML model.

Figure 6.5.2.1-1: The procedures for AnLF based error monitoring and handling

0.	The NWDAF containing AnLF subscribes to a (set of) trained ML model(s) associated with a (set of) Analytics ID(s) by invoking Nnwdaf_MLModelProvision_Subscribe service operation as described in step 1 of Figure 6.5.2-1.

1.	The NWDAF containing MTLF provides a (set of) trained ML model(s) associated with a (set of) Analytics ID(s) and may indicate a (set of) threshold(s) for accuracy of the trained ML model by invoking Nnwdaf_MLModelProvision_Notify service operation.

2a-2b.	The NWDAF containing AnLF generates a (set of) Analytics and provide the Analytics to the consumer NF according to the service operation as described in step 0 of Figure 6.5.2-1, and store the provided Analytics to measure the accuracy compared to the actual data. The NWDAF containing AnLF may request that the ADRF stores the events and data used for generating analytics by triggering Nadrf_DataManagement_StorageRequest.

3.	The NWDAF containing AnLF continues to collect actual events and data related to the provided analytics during the analytics target period by triggering the procedures in clause 6.2 of TS 23.288 [5]. The NWDAF containing AnLF may request that the ADRF stores the actual events and data related to the provided analytics during the analytics target period by triggering Nadrf_DataManagement_StorageRequest.

4.	The NWDAF containing AnLF measures the accuracy of the provided analytics at step 2a-2b by compared to the collected events/data at step 3.

5.	When the measured accuracy at step 4 is lower than the target accuracy of the analytics on the NWDAF containing AnLF and/or lower than the threshold for accuracy of the provided ML model, if provided at step 1, the NWDAF containing AnLF sends the errors by invoking Nnwdaf_MLModelProvision_Subscribe with the indication of cause = "inaccuracy". If the NWDWAF containing AnLF requests to store the actual events and data related to the provided analytics at ADRF at step 3, the AnLF may include ADRF information (i.e., ADRF ID, Storage Transaction Identifier, Notification Correlation ID) in Nnwdaf_MLModelProvision_Subscribe operation.

6.	The NWDAF containing MTLF may retrieve the events and data used for generating analytics and, the actual events and data related to the provided analytics during the analytics target period from the ADRF by invoking Nadrf_DataManagementRetrievalRequest service operation. The NWDAF containing MTLF may re-evaluate the provisioned ML model by compared the retrieved events and data to the trained data.

NOTE:	Step 6 can be triggered by the NWDAF containing MTLF internal logic or when the NWDAF containing MTLF get notification from ADRF, i.e., Nadrf_DataManagement_RetrievalNotify in which includes Fetch Instruction and the Notification Correlation ID.

6.5.2.2	Procedures for MTLF based error monitoring

Figure 6.5.2.2-1 describes the procedures that NWDAF containing MTLF monitors expected error on the analytics by the provisioned ML model.

Figure 6.5.2.2-1: The procedures for MTLF based expected error monitoring

0a.	The NWDAF containing MTLF trains an ML model with a set of trained data and stores the set of trained data or the characteristics of trained data (e.g. mean, variance, etc.) for step 3.

0b.	The NWDAF containing AnLF subscribes to a (set of) trained ML model(s) associated with a (set of) Analytics ID(s) by invoking Nnwdaf_MLModelProvision_Subscribe service operation as described in step 1 of Figure 6.5.2-1. If the NWDAF containing AnLF stores the events and data used for generating an Analytics at the ADRF, the NWDAF containing AnLF may provide the ADRF information (i.e., ADRF ID, Storage Transaction Identifier, Notification Correlation ID).

1.	The NWDAF containing MTLF provides a (set of) trained ML model(s) associated with a (set of) Analytics ID(s) to the NWDAF containing AnLF by invoking Nnwdaf_MLModelProvision_Notify service operation.

2.	The NWDAF containing MTLF discovers date source NFs by exploiting ML Model Filter Information and Target of ML Model Reporting, and collects actual events and data related to the analytics for the ML model during the ML Model Target Period by triggering the procedures in clause 6.2 of TS 23.288 [5]. If the NWDAF containing AnLF provides ADRF information at step 0b, the NWDAF containing MTLF may retrieve the events and data for generating an Analytics from the ADRF.

NOTE:	Step 6 can be triggered by the NWDAF containing MTLF internal logic or when the NWDAF containing MTLF get notification from ADRF, i.e., Nadrf_DataManagement_RetrievalNotify in which includes Fetch Instruction and the Notification Correlation ID.

3.	The NWDAF containing MTLF measures the differences between the set of trained data at step 0a and the collected events/data at step 2.

6.5.2.3	Procedures for MTLF to detect dynamics on the analytics

Figure 6.5.2.3-1 describes the procedures that NWDAF containing MTLF detect crucial changes in 5GS which impact on the ML model provision.

Figure 6.5.2.3-1: The procedures for detection of dynamics on the analytics

0a.	The NWDAF containing MTLF may have classes of 5GS state, which consist of e.g. set of configurations related to Target of Analytics Reporting, set of applied policies related to Target of Analytics Reporting, and set of expected events related to Target of Analytics Reporting, for ML Model provision, and may apply it to ML model selection for a certain analytics.

NOTE 1:	The classes of 5GS state may be set by operator's configuration or may be determined by the internal logic of NWDAF containing MTLF and it is out of SA2 scope.

0b.	The NWDAF containing AnLF subscribes to a (set of) trained ML model(s) associated with a (set of) Analytics ID(s) by invoking Nnwdaf_MLModelProvision_Subscribe service operation as described in step 1 of Figure 6.5.2-1.

1.	The NWDAF containing MTLF provides a (set of) trained ML model(s) associated with a (set of) Analytics ID(s) to the NWDAF containing AnLF according to the criteria and the classes of 5GS state by invoking Nnwdaf_MLModelProvision_Notify service operation.

2a-2d.	The NWDAF containing MTLF subscribes at least one of the services which provide the notification of changes on 5GS state which was set by the classes at step 0a (e.g. occurrence of changes on UE subscription data, occurrence of changed on UE policy, occurrence of changed on access and mobility related policy, occurrence of changed on session management related policy, configuration update by OAM, etc.) for the specific targets for analytics (e.g. the combination of S-NSSAI(s), UE IDs (SUPI), UE group ID(s), application ID(s), PDU Session ID(s), Serving AMF ID(s), Serving SMF ID(s), etc.). For details, the NWDAF containing MTLF may subscribe to Npcf_EventExposure_Subscribe operation with a new Event ID(s), i.e., UE policy update, AM policy update, and SM policy update. When the PCF invokes Npcf_UE/AM/SMPolicyControl_Update_response or Npcf_UE/AM/SMPolicyControl_UpdateNotify service operation, the new event ID(s) of PCF notifies the occurrence of UE, AM, and SM policy changes to the NWDAF containing MTLF, respectively. Also, the NWDAF containing MTLF may further subscribe to Nudm_SDM_Subscribe service in order to track the occurrence of the changes in UE subscription data (e.g., Access and Mobility Subscription data, Session Management Subscription data, Slice Selection Subscription data, etc.). UDM may subscribe to notifications from UDR for tracking the occurrence of changes on UE subscription data by Nudr_DM_Subscribe.

3a-3c.	According to the subscription at step 2a-2d, the NWDAF containing MTLF receives the information of the occurrence of changes on 5GS state which was set by the classes at step 0a.

4.	In order to evaluate the impacts of 5GS state changes at step 3a-3c on the performance of provisioned ML model, the NWDAF containing MTLF may collect the event and data related to the ML model and the analytics during ML Model Target Period by triggering the procedures in clause 6.2 of TS 23.288 [5].

5.	The NWDAF containing MTLF evaluates the inconsistency of the provisioned ML model for the changed 5GS state at step 3a-3d with compared to the classes of 5GS state at step 0a and/or the actual data and events at step 4. According to the results of evaluation, the NWDAF containing MTLF may update the classes of 5GS state for ML model provision.

6.5.3	Impacts on existing nodes and functionality

NWDAF containing AnLF:

-	AnLF can measure the error between the provided analytics and actual data/events.

-	AnLF is able to store the events and data related to the provided analytics during the analytics target period in ADRF and to send ADRF information to the MTLF.

-	AnLF is able to store events and data used for generating an Analytics in ADRF and to send ADRF information to the MTLF.

NWDAF containing MTLF:

-	MTLF can measure differences between the trained data set for the provisioned ML model and the actual data/events.

-	MTLF should consider the errors on analytics, the differences between actual data/events and the trained data set, and/or the impact of the configuration updates, policy changes and various unexpected events to improve ML model provision services.

-	Some additional parameters will be added to Nnwdaf_MLModel_Provision_Subscribe/Notify service operation to support the procedure as described in clause 6.5.2.1.

NF:

-	In order to track the occurrence of changes on 5GS state, PCF supports new event ID (s), i.e., UE policy update, AM policy update, SM policy update for event exposure.

6.6	Solution #6: Correctness improvement of NWDAF by determining ML model performance

6.6.1	Description

This solution addresses Key Issue #1 "How to improve correctness of NWDAF".

After the Analytics consumer NF consumes the analytics from the NWDAF containing AnLF, some network data are generated. The NWDAF shall collect these network data from the Analytics consumer NF after the Analytics consumer NF consumed the Analytics results. The network data are used to detect that improving the correctness of an Analytics ID is needed. And the network data will be used for the NWDAF to determine the ML model performance and/or re-train the ML model.

This solution provides the following functionalities:

-	The NWDAF collects network data of consumer NFs to support NWDAF perceive the correctness and ML model performance.

-	The NWDAF calculates network data to evaluate ML model performance and trigger further ML model training to improve the accuracy of NWDAF.

-	The NWDAF can reduce the signalling load of data collection through data collection function NF such as DCCF.

6.6.2	Procedures

The following figure presents an example procedure of correctness improvement of NWDAF by determining ML model performance.

Figure 6.6.2-1: Procedure for correctness improvement of NWDAF by determining ML model performance

1.	Steps 1 specified in TS 23.288 [5] are reused to perform network data analytics for the analytics consumer and data collection for the data consumer.

2-3.	After the analytics consumer NF consumes the analytics, the NWDAF containing AnLF collects the network data by invoking the Nnf_EventExposure_Subscribe service operation or via DCCF by invoking the Ndccf_DataManagement_Subscribe service operation, the network data may include the data from analytics consumer NF and/or data source. This step is skipped if the NWDAF containing AnLF was subscribed to the analytics consumer NF and/or data source directly or via DCCF in step 1.

4.	The NWDAF containing AnLF determines ML model performance (e.g. accuracy, precision or recall) by comparing the network data and the analytics provided to the Analytics consumer NF in step 1.

The NWDAF containing AnLF determines whether further training of the ML Model is needed based on the ML model performance and internal strategies.

If further training of the ML Model is needed, the NWDAF containing AnLF subscribes ML model associated with the Analytics ID in step 1 by invoking the Nnwdaf_MLModelProvision_Subscribe to get the re-trained ML model, the subscribe information may include the ML model performance.

The NWDAF containing MTLF determines that further training is needed, this NWDAF may initiate data collection from NFs, (e.g. AMF/DCCF/ADRF), UE Application (via AF) or OAM as described in TS 23.288, to re-train the ML model.

In addition, the NWDAF containing MTLF may collect the network data from the NWDAF containing AnLF for re-training the ML model.

The NWDAF containing MTLF notifies the NWDAF containing AnLF with the re-trained ML Model Information (containing a (set of) file address of the trained ML model) by invoking Nnwdaf_MLModelProvision_Notify service operation.

6.6.3	Impacts on existing nodes and functionality

NWDAF containing AnLF:

-	Supports collecting the network data after the analytics consumer NF consumes the analytics.

-	Supports determining ML model performance.

-	Supports triggering further training of the ML Model based on the ML model performance.

NWDAF containing MTLF:

-	Supports further training of the ML Model based on the ML model performance.

6.7	Solution #7: Enhancements to NWDAF analytics services

6.7.1	Description

In general, the ability for a service consumer to use a performance score to select an NWDAF Analytics and/or ML model may help such service consumer to obtain the desired level of correctness. This solution allows to rate ML models and/or NWDAF analytics provided by different producers (or by the same producer offering multiple ML models for the same NWDAF analytics).

In particular, an analytics consumer is allowed to rate an analytics service (i.e. Analytics ID) from an NWDAF (see procedure in clause 6.7.2.1), whereas an NWDAF containing AnLF is enabled to rate an ML model provided by an NWDAF containing MTLF (see procedure in clause 6.7.2.2). In this solution, a consumer from the same vendor of the analytics service or model provisioning service may rate the analytics service or the used ML model, while the entity producing/exposing the analytics service or ML model is not allowed to self-rate it. For example, an analytics consumer from the same vendor as the NWDAF containing AnLF producer can rate the analytics service, while self-ratings from the NWDAF containing AnLF are not accepted.

The solution leverages on the introduction of:

-	A Trusted Rating Logical Function (TRLF) that manages the rating provided by the service consumers. Furthermore, it ensures that only verified consumers (i.e. consumers that really have had access to the ML model and/or Analytics services) can rate the ML model and/or the Analytic service. For example, it prevents rating (of its own) by the producer of the ML model and/or analytics service while an analytics consumer from the same vendor is still able to rate the analytics service. For example, NWDAF(MTLF) that has produced the ML model is forbidden to rate such ML model, while NWDAF(AnLF) from the same vendor could rate the ML model if it has used the ML model.

-	A rating format that includes key information regarding the usage of the Analytics service provided or not by using an ML model and that can be exploited by other consumers as well as by other producers/vendors.

6.7.2	Procedures

6.7.2.1	Analytics consumer rating an NWDAF analytics provided by an NWDAF containing AnLF

Figure 6.7.2.1-1: Procedure for analytics consumer rating an NWDAF analytics provided by an NWDAF containing AnLF

Pre-condition: NF profile of the NWDAF in NRF contains a metric that should be utilized to rate the analytics service. A metric is associated to each analytics service (i.e. Analytics ID).

1.	An Analytics Consumer sends a discovery request to NRF looking for NWDAFs providing the Analytics ID(s) and other input parameters as specified in clause 6.1.3 of TS 23.288 [5].

2.	The NRF returns to the Analytics consumer the list of available NWDAFs matching the filter parameters along with the metrics to rate the analytics.

3.	The Analytics Consumer requests, for the discovered NWDAFs, the ratings of the Analytics ID from the Trusted Rating Logical Function (TRLF) through a Ntrlf_RatingDiscovery service. In the request, the Analytics Consumer specifies the NWDAF(s) and the Analytics ID(s). The Analytics Consumer sets the "rating aggregation level" request parameter according to its preferred value: "Global Average" to receive an aggregated model rating (i.e. a single value), "average per vendor" to receive a single average value for each vendor which issued a rating, or "detailed" in case it is interested to receive the rating from each service consumer which casted a rating.

The aggregated rating is a value, e.g. between 0 (very bad performance) to 5 (very good performance) derived by the TRLF by an average over all ratings. Along with the aggregated rating, the Analytics Consumer receives also the total number of ratings submitted, so that the consumer can derive the accuracy of the rating.

The Analytics consumer may also implement a local cache for such ratings, in order to avoid the need to query TRLF for each NWDAF discovery request. In this case, steps 2 and 3 may be skipped for future requests.

4.	The TRLF returns to the Analytics Consumer the requested ratings according to the specified "rating aggregation level". Based on the "average per vendor" or "detailed" model rating, the analytics consumer can identify how ratings were casted by each vendor.

5.	The Analytics consumer selects the NWDAF providing the best performance for the specific use case and scenario. The Analytics consumer requests the analytics service to the selected NWDAF specifying also its Consumer ID comprising the NF (instance or Set) ID and Vendor ID.

6.	The NWDAF generates a token that can be used by the Analytics consumer to rate the analytics service.

NOTE:	The normative aspects about token generation, and how it is used for verification and communicated to other NFs should be carried out in coordination with SA3.

7.	The NWDAF sends through the Ntrlf_AnalyticsServiceConsumed service to the TRLF information about the Consumer ID, Analytics ID, information on the ML model used for producing the analytics (if any), its own NWDAF (instance or Set) ID and the token generated for the Analytics consumer. In this way, the TRLF can associate the rating from the Consumer to the analytics service provided by the NWDAF and, implicitly, to the ML model used to generate it in case the analytics service is based on an ML model.

8.	The TRLF sends an acknowledgement to the NWDAF.

9.	The NWDAF sends the analytics response to the Analytics consumer along with the token generated for allowing only verified consumers (i.e. only the ones that really have consumed the service) to evaluate the analytics service.

In case the analytics consumer subscribed to the analytics service, the token is valid for the entire subscription duration and the consumer may update its rating by sending another Ntrlf_AnalyticsRating request. Once the subscription is terminated, the NWDAF shall inform the TRLF about it, such that only a final rating can be provided by the consumer after which the token is revoked.

10.	The analytics consumer evaluates the performance of the analytics service utilizing the metric obtained by NRF during the discovery procedure.

11.	The analytics consumer through the Ntrlf_AnalyticsRating service sends its rating to the TRLF. The request also includes the Consumer ID of the analytics consumer and the received token.

12.	The TRLF, in case the token matches and the analytics consumer is not the model producer, accepts and updates the rating. The TRLF stores the rating per Analytics ID and for each Consumer ID.

13.	The TRLF sends to the analytics consumer a confirmation regarding the update of the rating.

6.7.2.2	NWDAF containing AnLF rating an ML model provided by an NWDAF containing MTLF

This procedure, depicted in Figure 6.7.2.2-1, enables an NWDAF containing AnLF to rate an ML model received by NWDAF containing MTLF.

Figure 6.7.2.2-1: Procedure for NWDAF containing AnLF rating an NWDAF analytics provided by an NWDAF containing MTLF

1.	The NWDAF containing AnLF requests, for the NWDAFs that support the Analytics ID for the desired AOI) the rating(s) of the employed ML model(s) from the Trusted Rating Logical Function (TRLF) through a Ntrlf_RatingDiscovery service. In the request, the NWDAF containing AnLF specifies the NWDAF(s) and the Analytics ID(s). The NWDAF containing AnLF sets the "rating aggregation level" request parameter according to its preferred value: "Global Average" to receive an aggregated model rating (i.e. a single value), "average per vendor" to receive a single average value for each vendor which issued a rating, or "detailed" in case it is interested to receive the rating from each service consumer which casted a rating.

2.	The TRLF returns to the NWDAF containing AnLF the requested ratings per model ID and per Analytics ID, according to the specified "rating aggregation level". Based on the "average per vendor" or "detailed" model rating, the analytics consumer can identify how ratings were casted by each vendor.

3.	The NWDAF containing AnLF selects the NWDAF containing MTLF providing the required performance of the ML model for the specific use case and scenario. The NWDAF containing AnLF subscribes to ML model provisioning service to the selected NWDAF specifying also its Consumer ID comprising the NF (instance or Set) ID and Vendor ID.

4.	The NWDAF containing MTLF generates a token that can be used by the NWDAF containing AnLF to rate the ML model.

5.	The NWDAF sends through the Ntrlf_AnalyticsServiceConsumed service to the TRLF information about the Consumer ID, Model ID and version used for producing the analytics, its NWDAF ID and version and the token generated for the NWDAF containing AnLF.

6.	The TRLF sends an acknowledgement to the NWDAF containing MTLF.

7.	The NWDAF containing MTLF sends the subscription notification to the NWDAF containing AnLF along with the token generated.

8.	The NWDAF containing AnLF evaluates the performance of the ML model utilizing the metric obtained by NRF during the discovery procedure.

9.	The NWDAF containing AnLF through the Ntrlf_AnalyticsRating service sends its rating to the TRLF. The request also includes the Consumer ID of the NWDAF containing AnLF and the received token.

10.	The TRLF, in case the token matches and the NWDAF containing AnLF is not the model producer, accepts and updates the rating. The TRLF stores the rating per Analytics ID and for each Consumer ID.

11.	The TRLF sends to the NWDAF containing AnLF a confirmation regarding the update of the rating.

Editor's note:	It is FFS how to rate the ML model and analytics.

6.8	Solution #8: NWDAF assisted service type detection

6.8.1	Description

This solution is for Key Issue#2: NWDAF-assisted application detection.

The application detection is an important mechanism for the operator e.g. to optimize its user’s service experience; relieve the load of the network function; etc.

-	For example, for XR service delivered over HTTPS, it is not able to leverage DPI to get XR related information to assist smart scheduling as proposed in XR study and it is interesting to discuss how to leverage AI/ML e.g. DFI to get some XR related information.

-	Another example, it is useful for the operator to monitor which application (e.g. a video service which is relatively not urgent) lead to the heavy load of an NF or a network slice then make decisions to relieve the NF load or NSI Load e.g. reduce the Session-AMBR for the PDU Session who serves the application.

Traditionally, the application traffic detection in user plane is depend on DPI (deep packet inspection) function installed in UPF. However, the DPI in only suitable for plaintext traffic (i.e. original readable text in the packet) but not for the encrypted traffic.

To help the encrypted application traffic detection, as defined in clause 6.1.2.3 of TS 23.503 [4], ASP may provide the PFD (packet flow description) information including Application ID, 3-tuple(s) etc. to the UPF via NEF (PFDF)/SMF. Then the UPF can determine the Application ID for the encrypted traffic based on the PFD information and the 5-tuple of the encrypted traffic. However, it is the ASP who determines if the PFD information is provided or not, which may be out of control for the Operator.

If no any PFD information from the ASP, it is possible for NWDAF to derive Service Type analytics (including e.g. Service Type and its corresponding IP 3-tuple and/or Traffic characteristics) to help the traffic detection in UPF.

This solution proposes to define a new terminology i.e. Service Type including Browsing, Video Streaming, Audio Streaming, File Sharing, Multimedia, Interactive Messages, Video call over IP, Voice call over IP and so on. Alternatively, the definition of the current Application ID in clause 3.1 of TS 23.503 [4] can be enhanced to support these service types.

Editor's note:	Whether there is a need for a Service Type or the Application ID can cover is FFS.

As a potential consumer NF, the PFDF may subscribe or request the Service Type analytics and locally store the analytics as new PFD information, and then the PFDF pushes the new PFD information to SMF and UPF to identify the Service Type of the traffic flows based on the new PFD information and the 3-tuple of traffic in the UPF.

-	For any UE, based on operator policy, PCF may trigger the Service Type detection in a specific area of interest and requests the NEF(PFDF) to push the new PFD information (including the Service Type related information) to the UPFs located in the AoI. Then the UPF may report the service type detection result for this AoI to PCF for QoS control.

-	For a single UE, PCF may get the service type detection requirement per UE/per AF session from AF, then SMF may pull the PFD information from the NEF(PFDF) to the UPF. Then the UPF may report the service type detection result for this UE to PCF for QoS control.

Editor's note:	How the new PDR is activated and the reporting from UDR to PCF is done is FFS.

Editor's note:	How the Service Type detection and Service Type QoS control coexist with the existing SDF detection and QoS control define in TS 23.503 [4] is FFS.

The input data for the Service Type analytics is shown in Table 6.8.1-1.

Table 6.8.1-1: Service flow level Data from UPF and/or AF related to a particular Service Type

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | UPF, AF | A time stamp associated with the collected information. |
| 3-tuple | UPF, AF | protocol, server side IP address and port number for a service flow |
| Service Type | AF | Service Type of the service data flow |
| Duration | UPF, AF | Duration of the service flow level data |
| UL/DL Bit Rate | UPF, AF | The observed bit rate for UL direction; and The observed bit rate for DL direction. |
| UL/DL Packet Delay | UPF, AF | The observed Packet delay for UL direction; and The observed Packet delay for the DL direction. |
| Number of UL/DL packet transmission | UPF, AF | The observed number of UL/DL packet transmission. |
| Size of UL/DL packet retransmission | UPF, AF | The observed size of UL/DL packet transmission. |

Editor's note:	Whether the Service Type or the AF-Application ID is reported by the AF and why a new identifier is needed is FFS.

NOTE 1:	How to collect the QoS flow level Network Data from UPF is depend on the conclusion of FS_UPEAS.

To collect the UL/DL Packet Delay, the current QoS Monitoring Event (including end to end delay i.e. Uu+N3/N9 delay for specific QoS flow or for specific PDU session) by UPF as defined in clause 5.2.26.2 of TS 23.502 [3] can be re-used.

NOTE 2:	The current Packet delay detection and reporting in R17 is only supported for URLLC but how NWDAF collects the UL/DL Packet Delay from UPF will be defined in FS_UPEAS.

Editor's note:	Extensive reporting of all traffic flows may conflicts with requirement to avoid extra UPF load in the key issue. How to avoid the degradation of UPF Load because of the extensive reporting of all traffic flows should be aligned with the conclusion of KI#2: Support UPF expose information to other NFs in FS_UPEAS.

With the input data, NWDAF uses the ML algorithm (e.g. K-Means) to cluster the data set to derive one or more 3-tuple(s) for a Service Type. The statistics and prediction types of Service Type analytics are shown in Table 6.8.1-2 and Table 6.8.1-3, respectively.

Table 6.8.1-2: Service Type statistics

| Information | Description |
| --- | --- |
| List of Service Types (1..max) |  |
| > Service Type | Service Type (including i.e. Browsing, Video Streaming, Audio Streaming, File Sharing, Multimedia, Interactive Messages, Video call over IP, Voice call over IP) of the traffic in UPF. |
| >> IP 3-tuple | One or more IP 3-tuples for the Service Type |
| >> Traffic characteristics (NOTE) | Traffic characteristics (including UL/DL Bit Rate, UL/DL Packet Delay, Number of UL/DL packet transmission, Size of UL/DL packet retransmission, etc.) of the Service Type |
| NOTE: Traffic characteristics helps the Service Type detection i.e. the UPF may determine a Service Type for a real-time traffic information if the real-time traffic information and the traffic characteristics for the Service Type is matched. |  |

Table 6.8.1-3: Service Type predictions

| Information | Description |
| --- | --- |
| List of Service Types (1..max) |  |
| > Service Type | Service Type of the traffic in UPF. |
| >> IP 3-tuple | One or more IP 3-tuples for the Service Type |
| >> Traffic characteristics (NOTE) | Traffic characteristics (including UL/DL Bit Rate, UL/DL Packet Delay, Number of UL/DL packet transmission, Size of UL/DL packet retransmission, etc.) of the Service Type |
| > Confidence | Confidence of this prediction |
| NOTE: Traffic characteristics helps the Service Type detection i.e. the UPF may determine a Service Type for a real-time traffic information if the real-time traffic information and the traffic characteristics for the Service Type is matched. |  |

6.8.2	Procedures

Figure 6.8.2-1: Procedure for Service Type analytics provided to an NF

1.	The NF sends Nnwdaf_AnalyticsInfo_Request or Nnwdaf_AnalyticsSubscription_Subscribe (Analytics ID="Service Type", Target of Analytics Reporting, Analytics Filter Information, Analytics Reporting Information = ("Reporting Thresholds" and Analytics target Period(s))) to the NWDAF.

2.	The NWDAF collects service flow level data from UPF as listed in Table 6.8.1-1.

NOTE:	How to collect the service flow level data from UPF is depend on the conclusion of FS_UPEAS.

3.	The NWDAF collects service flow level data from AF as listed in Table 6.8.1-1.

4.	The NWDAF derives the requested analytics.

5.	The NWDAF sends Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify (Service Type analytics).

6-7.	A change of user plane traffic information which is reported by UPF or AF, and is notified to NWDAF.

8.	The NWDAF derives new analytics taking into account the most recent data collected.

9.	The NWDAF provides a notification for the new analytics using Nnwdaf_AnalyticsSubscription_Notify service operation.

6.8.3	Impacts on existing nodes and functionality

NWDAF:

-	Support for a new Analytics ID i.e. Service Type analytics.

-	Support subscribing for service flow level data from UPF and/or from AF and providing Service Type analytics to NF.

UPF/AF:

-	Report service flow level data to NWDAF.

6.9	Solution #9: NWDAF-assisted application detection

6.9.1	Description

This solution is proposed to address Key Issue #2: NWDAF-assisted application detection.

To differentiate an application traffic handling, the first step is to distinguish application traffic from other traffics. In 5G network, an application can be distinguished by a set of packet headers (SDFs) or application IDs. The most common approach to detect traffic is use a set of packet headers which usually contains source and destination address. The application ID can be used for referring to the UPF's specific application detection filter, and AF may provide PFDs according to clause 4.18 of TS 23.502 [3] to update application detection filter information associated with the application ID. A PFD contents could contain flow description (service-side 3-tuple), URL, and domain name/protocol information. When an AF delivers PFD to NEF (PFDF), it will be distributed to SMFs and UPFs to enable flow detection according to clause 5.8.2 of TS 23.501 [2].

However, as indicated by the KI description, the ASP may provide an initial PFD information but does not update it in time or does not update it anymore, then it is unknown how the UPF detect the application traffic.

The objective of this solution is to to provide analytics on application traffic. The proposed solution provides analytics including new PFDs for unknown applications and updated PFDs for known applications by extracting network traffic signature and inferring application identity. To realize this solution, one of the main features is 1) collect measurement of an application to extract statistical characteristics, 2) collect payload of packets to extract payload characteristics (such as domain name contained in the payload). The baseline usage of the generated analytics is to store the captured application characteristics as a PFD, and the PFD is used by SMF and UPF to detect an application defined as TS 23.502 [3].

Editor's note:	How new PFDs for unknown applications can be used is FFS.

The NWDAF could collect the current and historical PFD information (including Application ID, historical IP 3-tuple, historical URL, historical Domain name information) from the NRF via NEF (PFDF) and ADRFrespectively.

Based on PFD information from ADRF and NRF and traffic information from UPF, new PFD information (including Application ID, new IP 3-tuple, new URL, new Domain name information) for the existing Application ID could be derived by the NWDAF. The new PFD information provided by the NWDAF can be used by the NEF for provisioning SMF/UPF.

In case that there is overlapping PFD information provided by NWDAF and provided by AF, the latter will take precedence over the PFD information provided by NWDAF.

The below is the principles of the solution:

-	A consumer (e.g. NEF) requests to the NWDAF to provide PFDs for newly detected applications.

-	The NWDAF requests to provide information on SDF that does not match an installed PDR at the SMF/UPF.

-	The NWDAF analyses the collected data to generate unique traffic patterns, and provides it to the consumer (i.e. NEF).

-	The consumer i.e. NEF stores PFD associated with an application ID to enable the detection of application traffic in the future (out of scope).

6.9.2	Input Data

NWDAF collects QoS flow related data from SMF for a specific S-NSSAI, DNN, and UE. The detailed data are described in Table 6.9.2-1.

Table 6.9.2-1: input data to detect new application from NFs

| Information | Source | Description |
| --- | --- | --- |
| SUPI | SMF / UPF | UE ID for the UE who uses the application. |
| S-NSSAI | SMF | Identifies the Network Slice for which analytics information is provided. |
| DNN | SMF | Identifies the data network name (e.g. internet) for which analytics information is provided |
| Start/end time | UPF | Start and end time of traffic detection |
| Data volume | UPF | Measured data traffic volume (per UL/DL) |
| Data duration | UPF | Measured average data traffic (packets) duration (per UL/DL) |
| QoS flow Bit Rate | UPF | The observed bit rate (per UL/DL) |
| Packet transmission | UPF | The observed average number of packet transmission (per UL/DL) |
| IP 3-tuple | UPF | to Identify a service flow of the UE who uses the application |
| Packet's URL | UPF | URL is extracted from the inspected user plane packets. |
| Packet's Domain Name | SMF/UPF | The domain name is extracted from the inspected user plane packets in a QoS flow |
| Size of packets | SMF/UPF | Average size of packets |
| PFD Information | ADRF | Historical PFD Information stored in ADRF |
| > Application ID | ADRF | Identification of the application that refers to one or more application defection filters. |
| > IP 3-tuple | ADRF | Including protocol, server side IP address and port number |
| > URL | ADRF | the significant parts of the URL to be matched, e.g. host name |
| > Domain Name | ADRF | a Domain name matching criteria and information about applicable protocol(s) |
| PFD Information | NEF | PFD Information stored in the NRF and retrieved by NEF, as defined in clause 6.1.2.3.2, TS 23.503 [4] |
| > Application ID | NEF | Identification of the application that refers to one or more application defection filters. |
| > IP 3-tuple | NEF | Including protocol, server side IP address and port number |
| > URL | NEF | the significant parts of the URL to be matched, e.g. host name |
| > Domain Name | NEF | a Domain name matching criteria and information about applicable protocol(s) |

NOTE:	Extensive reporting of all traffic flows may conflict with requirement to avoid extra UPF load in the key issue The issue regarding the performance of UPF user plane traffic handling will depend on the outcome of FS-UPEAS SID.

6.9.3	Output Analytics

The output analytics of NWDAF is defined in Table 6.9.3-1.

The output analysis can be used to provision new PFDs for known applications, or define new PFDs for new applications, not known to NEF yet. Upon receiving the analytics including new PFDs information from the NWDAF, the NEF can assign a PFD ID for the unknown application. And, the assigned PFD ID can be used to identify/audit/report the unknown application.

Table 6.9.3-1: Output for new application detection

| Information | Description |
| --- | --- |
| Application ID | Application ID of the application that refers to the application detection filter (Flow descriptor, URL or Domain name information) stored in the UDR. In case of detection of unknown application, this parameter has a null value. |
| S-NSSAI | Identifies the Network Slice for which analytics information is provided. |
| DNN | Identifies the data network name (e.g. internet) for which analytics information is provided |
| Flow descriptor | Flow descriptor containing 3-tuple, server side (destination address, port, and protocol) |
| URL | the significant parts of the URL to be matched, e.g. host name defined in |
| Domain name information | a Domain name matching criteria and information about applicable protocol(s). |

6.9.4	Procedures

The procedure depicted in 6.9.4-1 shows new application detection analytics.

Figure 6.9.4-1: A procedure to extract application characteristics

1.	The consumer NF subscribes to the NWDAF to request analytics for application detection. This subscription maybe is triggered by local configuration or OAM. The Analytics Filter Information may optionally include the UE ID, S-NSSAI and/or DNN.

2.	The NWDAF fetches currently stored PFD information from NEF(PFD) and historical data from ADRF (not shown in the figure).

3.	The NWDAF collects session related information from the UPF about URL, Domain name part, and IP 3-tuples of packets from the SDF not matching installed PDRs.

4.	The NWDAF derives PFD analytics.

5.	The NWDAF notifies the analytics consumer NF with outputs.

6.9.5	Impacts on existing nodes and functionality

NWDAF:

-	Needs to provide a new analytic for application detection information.

-	Needs to process user plane data for extracting traffic characteristics.

UPF:

-	Needs to report URL, Domain name part, and IP 3-tuples of packets from unknown application to NWDAF.

-	Needs to provide requested sessions statistics including data volume, data duration, QoS flow bit rate and packet transmission.

NEF (PFDF):

-	Supports to consume new PFD information from NWDAF.

Editor's note:	ADRF impacts are FFS.

6.10	Solution #10: Support for Data and Analytics Exchange in Roaming Case

6.10.1	Description

When two VPLMN and HPLMN expose data or analytics to each other, both PLMNs need the ability to control the amount of data exposed and to abstract or hide network-internal aspects based on user consent, operator policy, regulatory constraints and/or roaming agreements.

The solution following the following assumptions and principles:

-	The hPLMN may impose constraints on the type or amount of data collected. The constraints may differ for different vPLMNs.

-	The vPLMN may impose constraints on the type or amount of data collected. The constraints may differ for different hPLMNs.

-	The data collection must honor user data consent for user related data collection.

The solution proposes a Gateway Exposure Function (GEF) in a PLMN, which is responsible for data/analytics exposure to other PLMNs. The GEF is provisioned with constraints on the type and amount of data exposed to each PLMN according to roaming agreement. The GEF is also responsible for enforcing obtaining the user consent for data collection where applicable.

When a data/analytics consumer in PLMN A (vPLMN/hPLMN) needs to collect data/analytics from PLMN B (hPLMN/vPLMN), the consumer contacts the GEF in PLMN A, then GEF in PLMN A contacts GEF in PLMN B, or the consumer contacts the GEF in PLMN B directly. The GEF in PLMN B discovers the data/analytics producer in PLMN B and collects the data/analytics. GEF is also responsible for data/analytics manipulation before sending to the other PLMN according to the regulatory constraints and agreement.

Editor's note:	It is FFS if the GEF is a standalone function or collocated with either NEF, DCCF or NWDAF.

6.10.2	Procedures

Figure 6.10.2-1: Data and Analytics Exposure via GEF when data/analytcis consumer is in vPLMN

Figure 6.10.2-1 shows the procedure when the data/analytics consumer is in the visiting PLMN (vPLMN), the NWDAF in the visiting PLMN (vNWDAF) needs to collect data/analytics from home PLMN (hPLMN) of the UE.

NOTE:	The UE can be one single UE, group of UE or any UE roaming to the VPLMN from the HPLMN.

It is assumed that both vGEF and hGEF have been provisioned, by their respective operators, with the policies and constraints for exporting data to or from roaming/home networks.

0.	GEF is registered itself in the NRF. The GEF profile may include a type to indicate data collection or analytics exposure.

1-2.	The analytics consumer in the vPLMN (e.g. an AMF) wants to subscribe an Analytics for an inbound roaming UE, the vNWDAF determines that it needs to collect data/analytics from hPLMN of the UE. For example, the data to be collected might be related to expected UE behaviour. The analytics consumer and vNWDAF obtain the hPLMN ID from the SUPI of the UE. In the analytics request in step 1, the analytics consumer may provide indication to the vNWDAF, that the analytics needs data from hPLMN of the UE, or the vNWDAF can determine that based on the Analytics ID.

There are 3 options to establish the connection between the vPLMN and HPLMN.

Option A:

3.	vNWDAF discovers vGEF via vNRF. vNRF provides the ID of vGEF that is responsible for collecting data/analytics from that PLMN.

Editor's note:	It is FFS on GEF discovery and selection.

4.	vNWDAF sends data/analytics subscription request to the vGEF. Similar services as Data management and analytics exposure service of NWDAF (defined in clause 7.2, 7.3 and 7.4 in TS 23.288[5] ) can be defined as the data and analytics subscription services provided by the GEF.

5.	vGEF discovers hGEF via vNRF and hNRF. vGEF provides HPLMN ID to vNRF. vNRF interacts with hNRF via N27 reference point as defined in TS 23.501[2].

6.	vGEF send data/analytics subscription request to the hGEF which is got in step 5. hGEF checks roaming agreements, hPLMN policies and regulatory constraints between hPLMN and the origin vPLMN to determine if the request can be accepted or must be rejected.

Option B:

3.	vNWDAF discovers vGEF via vNRF. vNRF provides the ID of vGEF that is responsible for collecting data/analytics from that PLMN.

4.	vNWDAF discovers hGEF via vNRF and hNRF taking into account the home PLMN ID and the request data or analytics exposure. The hNRF returns the ID of hGEF to the vNWDAF.

5.	vNWDAF sends data/analytics subscription request to the vGEF. The vNWDAF sends the ID of hGEF to the vGEF.

6.	vGEF send data/analytics subscription request to the hGEF. hGEF checks roaming agreements, hPLMN policies and regulatory constraints between hPLMN and the origin vPLMN to determine if the request can be accepted or must be rejected.

Option C:

3.	vNWDAF discovers hGEF via vNRF and hNRF taking into account the home PLMN ID and the request data or analytics exposure. hNRF provides the ID of hGEF that is responsible for collecting data/analytics from that PLMN to vNWDAF via vNRF.

4.	vNWDAF sends data/analytics subscription request to the hGEF. hGEF checks roaming agreements, hPLMN policies and regulatory constraints between hPLMN and the origin vPLMN to determine if the request can be accepted or must be rejected.

NOTE:	Which option(s) to be supported will be determined in the conclusion phase.

7-8.	If the checks in step 6 is OK, hGEF enforces user consent (for UE-related data collection) and collects data/analytics from the data/analytics sources in the hPLMN.

9.	hGEF manipulates the data/analytics according to the hPLMN policies and regulatory constrains and roaming agreements, e.g. remove some sensitive data, changes the granularity of the data, anonymization, formatting, etc.

10-11.	hGEF sends the data/analytics to vGEF and vGEF forwards to vNWDAF. vGEF may also manipulate the data/analytics to the format that the vNWDAF can understand.

12.	Alternatively, hGEF can send the data/analytics notification directly to the vNWDAF, if vGEF provides notification endpoint information of the vNWDAF in step 6.

When the analytics/data consumer is located in hPLMN, and the UE is roaming, the procedure shown in figure 6.10.2-2 applies instead. The difference from in figure 6.10.2-1 is step 3, which is now split into steps 3a and 3b. In step 3a the hNWDAF enforces the user consent with the hUDM. In step 3b the hNWDAF queries hUDM for retrieving the GUAMI of the AMF that is serving the user. This GUAMI contains the PLMN ID of the AMF, therefore, it can be used for determining whether the UE is roaming or not.

Figure 6.10.2-2: Data and Analytics Exposure via GEF when data/analytics consumer is in hPLMN

6.10.3	Impacts on services, entities and interfaces

Editor's note:	The impacts on services, entities and interfaces are FFS.

6.11	Solution #11: PDU session management in roaming scenarios using network analytics

6.11.1	Description

This solution addresses key issue #3: "Data and analytics exchange in roaming case".

PDU Session management in roaming scenarios can be improved using the data and analytics information exchange between HPLMN and VPLMN.

As specified in TS 23.502 [3], in the case of roaming, the AMF in the VPLMN determines if a PDU Session is to be established in LBO or Home Routing. For Home Routing, if the UE does not include an S-NSSAI in the PDU Session request, both a Serving PLMN S-NSSAI (in the Allowed NSSAI) and its corresponding HPLMN S-NSSAI values are selected by the AMF. Also, the AMF selects both an SMF in the Serving PLMN using the S-NSSAI of the Serving PLMN mapping to the S-NSSAIs of the HPLMN used for the PDU Session, and, additionally an SMF in the HPLMN using the S-NSSAI of the HPLMN used for the PDU Session. Here the selection of Network Slices (i.e. Serving PLMN S-NSSAI and its corresponding HPLMN S-NSSAI) and selection of SMFs (i.e. V-SMF and H-SMF) should take into account the status of Network Slices / NFs in both VPLMN and HPLMN. Therefore, analytics in HPLMN (i.e. Network Slice load level, NF load, etc.) can be sent to the VPLMN and used by the AMF in VPLMN for Network Slice and H-SMF selection for Home Routed PDU Session establishment.

6.11.2	Procedures

Figure 6.11.2-1 shows the procedure that V-NWDAF requests the Network Slice load level analytics in HPLMN from H-NWDAF, upon receiving an analytics information/subscription request from the AMF.

Figure 6.11.2-1: Procedure for providing HPLMN load level analytics to VPLMN

1.	AMF sends an Analytics request/subscribe (Analytics ID = Load level information, Analytics Filter Information = (VPLMN S-NSSAI, HPLMN S-NSSAI, HPLMN ID)) to V-NWDAF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe.

2.	V-NWDAF invokes Nnrf_NFDiscovery_Request (Expected Service Name = Nnwdaf_AnalyticsInfo (or Nnwdaf_AnalyticsSubscription), NF type of the expected NF = NWDAF, Analytics ID = Load level information, HPLMN ID, VPLMN ID, VPLMN S-NSSAI, HPLMN S-NSSAI, NF type of the NF service consumer = NWDAF) to an appropriate configured NRF in the VPLMN, and discovers the H-NWDAF using the procedure as described in clause 4.17.5 of TS 23.502 [3]. V-NWDAF sends an Analytics request/subscribe (Analytics ID = Load level information, Analytics Filter Information = HPLMN S-NSSAI) to H-NWDAF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe, based on the Analytics request/subscribe received from the AMF.

3-5.	H-NWDAF collects data from the NF(s) and/or OAM in HPLMN and derives the requested Network Slice / Network Slice instance load analytics information for the Network Slice identified by HPLMN S-NSSAI, as specified in clause 6.3 of TS 23.288 [5].

6.	H-NWDAF sends the HPLMN Network Slice / Network Slice instance load analytics information to the V-NWDAF using either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify.

7-9.	V-NWDAF collects data from the NF(s) and/or OAM in VPLMN and derives the requested Network Slice / Network Slice instance load analytics information for the Network Slice identified by VPLMN S-NSSAI, as specified in clause 6.3 of TS 23.288 [5]. These steps can be executed in parallel with step 3-6.

10.	V-NWDAF sends the HPLMN analytics information received in step 6, together with the VPLMN analytics information derived in step 9, to the AMF using either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify.

The AMF can use the HPLMN and VPLMN analytics information for Home Routed PDU Session establishment, as follows:

-	base on the HPLMN and VPLMN analytics information on Network Slice load, the AMF selects the Network Slices (i.e. VPLMN S-NSSAI and its mapped HPLMN S-NSSAI), and also the Network Slice instances in HPLMN and VPLMN if multiple VPLMN NSI IDs and/or HPLMN NSI IDs are available (e.g. received from the NSSF previously or configured) in the AMF for the selected Network Slices.

-	further, based on the HPLMN and VPLMN analytics information, e.g. the resource usage information in the Network Slice load analytics, or NF load analytics (using procedures similar to the above), the AMF selects an H-SMF in the selected Network Slice or Network Slice instance (if available) in the HPLMN, and a V-SMF in the selected Network Slice or Network Slice instance (if available) in the VPLMN, among the set of the SMF instance(s) returned by the NRF (as described in clause 4.3.2.2.3 of TS 23.502 [3]) or locally configured in the AMF.

6.11.3	Impacts on existing nodes and functionality

AMF: based on HPLMN and VPLMN analytics information, selects Network Slices and Network Slice instances (if available) in HPLMN and VPLMN, and selects the SMFs (i.e. V-SMF and H-SMF), for Home Routed PDU Session establishment in roaming scenarios.

V-NWDAF: requests HPLMN analytics information from the H-NWDAF.

H-NWDAF: provides HPLMN analytics information to the V-NWDAF.

6.12	Solution #12: DCCF and MFAF Relocation

6.12.1	Description

In scenarios where multiple DCCF and/or MFAF instances are deployed in the same network, there are some cases that the serving DCCF and/or MFAF may need to be changed, such as when a UE moves (e.g. due to idle mode mobility or a handover), the data or analytics source may change (e.g.: a new NWDAF, a new AMF, a new SMF). The new data source may not be in the area served by the current DCCF and/or the current MFAF. This solution proposes a way to change the DCCF and/or MFAF upon data source or analytics source change so the serving areas of the DCCF/MFAF and data source remain aligned.

The DCCF and MFAF relocation comprises the following steps:

-	When a DCCF coordinates data or analytics collection for a UE or a group of UEs, the DCCF subscribes to event notifications for UE mobility to outside of the serving area of the DCCF (for example area of interest as specified in TS 23.502 [3], clause 4.15.4.2). If an MFAF is used, the DCCF also subscribes to receive notifications of UE mobility to outside of the serving area of the selected MFAF.

-	When a DCCF coordinating data or analytics collection receives a mobility event notification:

It determines if the UE is still within the current DCCF serving area. If not, it determines a new DCCF (e.g. by querying the NRF). It then interacts with the new DCCF to transfer the UE DCCF context for the UE.

If an MFAF is used, the DCCF determines if the UE is within the current MFAF serving area. If not, it determines a new MFAF (e.g. by querying the NRF). The DCCF then interacts with the new MFAF to initiate a UE MFAF context transfer from the old MFAF or to establish a new UE MFAF context.

-	If there is a new MFAF or if there is a new DCCF and an MFAF is not used, the DCCF updates the Data Source with the new Notification Endpoint address. Subsequently, the data source sends notifications to the new DCCF or new MFAF.

MFAF UE Context information transferred or setup in the new MFAF may include information from on-going processing, such as buffered notifications and processed data, and configuration information such as Formatting Instructions, Processing Instructions, Data Consumer or Analytics Consumer information and MFAF notification information as described in TS 23.288 [5] clause 9.2.2.

DCCF UE Context information transferred to the new DCCF may include information from on-going processing, such as buffered notifications and processed data, and configuration information such as Service Operation, Analytics or Data Specification, Time Window, Formatting Instructions, Processing Instructions, Data Consumer or Analytics Consumer information and DCCF notification information as described in TS 23.288 [5] clause 8.2.2.

6.12.2	Procedures

Upon change of a source DCCF and/or MFAF instance, it is required to inform the related consumer of the event and possibly to update subscriptions between the DCCF and the consumer. The subscriptions can be renewed between the target DCCF instance and the consumer.

Figure 6.12.2-1: DCCF and/or MFAF Context Transfer

Figure 6.12.2-1 illustrates the procedure for DCCF and/or MFAF Context Transfer.

1.	AMF sends DCCF-1 a notification indicating that the UE is in a new area of interest.

2.	DCCF-1 determines that the UE is no longer in the area served by the DCCF-1 or in the area served by the MFAF-1.

3.	DCCF-1 may query the NRF to discover a DCCF and/or MFAF that can serve the UE in its new location. In the case of data subscriptions from multiple DCCFs, the DCCF-1 may preferably select a target DCCF that is already serving the consumer.

4.	If DCCF-1 determined in step 2 that it is no longer in the serving area of the UE, it selects a new DCCF (DCCF-2) to serve the UE. If the DCCF determined in step 2 that MFAF-1 is no longer in the serving area of the UE, it selects a new MFAF (MFAF-2) to serve the UE.

5.	If a new MFAF (MFAF-2) is selected, DCCF-1 sends a message to MFAF-2 requesting MFAF-2 to become the new MFAF and to retrieve the MFAF UE context from MFAF-1.

6.	If MFAF-2 receives the message in step 5, it retrieves the UE context from MFAF-1.

7.	MFAF-1 forwards to MFAF-2, notifications related to the UE (where formatting and processing may be performed by MFAF-2).

8.	MFAF-2 indicates to DCCF-1 that MFAF UE context transfer/setup is complete and provides MFAF-2 Notification Endpoint information to the DCCF-1.

9.	If a new DCCF (DCCF-2) is selected in step 4, DCCF-1 sends a message to DCCF-2 requesting DCCF-2 to become the new DCCF. If DCCF-2 accepts the transfer request:

a.	DCCF-2 retrieves the UE Context from DCCF-1.

b.	If an MFAF is used, DCCF-1 indicates to DCCF-2 the MFAF (MFAF-1 or MFAF-2).

c.	If an MFAF is not used, DCCF-2 provides DCCF-2 Notification Endpoint information in the response to DCCF-1.

d.	DCCF-2 confirms the context transfer from the DCCF-1.

10.	If a new DCCF (DCCF-2) is selected in step 4, DCCF-2 informs the MFAF (MFAF-1 or MFAF-2) that DCCF-2 is now the DCCF for the UE.

11.	DCCF-2 determines if the data source needs to be changed

12.	If the Data source is unchanged, DCCF-1 updates the Data Source with MFAF-2 Notification Endpoint Information (if there is a new MFAF-2), or DCCF-2 Notification Endpoint Information (if there is a new DCCF-2 and MFAF is not used) indicating that DCCF-2 is the new DCCF subscribing for data/analytics.

13.	If the Data source is changed, DCCF-2 subscribes to the relevant data source(s), if it is not yet subscribed to the data source(s) for the data required for the data subscription:

a.	If an MFAF is used, DCCF-2 indicates to Data source the MFAF (MFAF-1 or MFAF-2).

b.	If an MFAF is not used, DCCF-2 provides DCCF-2 Notification Endpoint information to Data source.

14.	DCCF-1 unsubscribes with the data source(s) that are no longer needed for the remaining data subscriptions.

Editor's note:	It is FFS whether and how to transfer the data subscription between DCCF and the data sources.

The consumer is informed by DCCF-1 that its subscription to DCCF-1 is now being handled by DCCF-2. In this message, the new Subscription Correlation ID, which was assigned by DCCF-2, is provided as the Subscription Correlation ID parameter and the old Subscription Correlation Id, which was allocated by DCCF-1, is provided as the Subscription Change Notification Correlation ID parameter.

Editor's note:	It is FFS how to do data collection transfer for the case of group UEs or any UE

6.13	Solution #13: NWDAF MTLF and NWDAF AnLF interoperability support for registration and discovery in 5GC

6.13.1	Description

This solution addresses KI#5: Enhance trained ML Model sharing.

The solution is based on the existing NWDAF discovery and selection as defined in TS 23.288 [5].

To discover an NWDAF containing MTLF via NRF by NWDAF containing AnLF belonging to a different vendor:

-	An NWDAF containing MTLF shall include ML model file serialization format supported in the ML model filter information for the trained ML model per Analytics ID(s).

-	During the discovery of NWDAF containing MTLF a consumer (i.e. an NWDAF containing AnLF) may include in the request ML model file serialization format supported per Analytics ID(s). The NRF returns one or more candidate for instances of NWDAF containing MTLF with the ML model file serialization format supported for the trained ML model per Analytics ID (s) as indicated by the NF consumer.

6.13.2	Procedures

Figure 6.13.2-1: NWDAF containing MTLF discovery and selection via NRF by NWDAF containing AnLF belonging to different vendors

NOTE:	The procedure described in this clause is based on the NWDAF discovery and selection as defined in TS 23.288 [5].

Editor's note:	How ML model privacy is supported when NWDAF containing MTLF and NWDAF containing AnLF belong to different vendors is in scope of SA WG3.

Editor's note:	Whether any other parameters apart from ML model file serialization format needs to be considered to support interoperability between NWDAF containing AnLF and NWDAF containing MTLF belonging to different vendors is for FFS.

An NWDAF containing MTLF shall send Nnrf_NFManagement_NFRegister request which include ML model file serialization format supported in the ML model filter information for the trained ML model per Analytics ID(s). The ML model file serialization format(s) included in the ML model Filter information indicates the supported ML model file serialization format(s) for the trained ML model(s) available at the NWDAF containing MTLF for the service consumer.

During the discovery of NWDAF containing MTLF a consumer (i.e. an NWDAF containing AnLF) may include in the request ML model file serialization format supported per Analytics ID(s) by the NWDAF containing AnLF.

The NRF returns one or more candidate for instances of NWDAF containing MTLF to the NF consumer and each candidate for instance of NWDAF with the ML model file serialization format supported for the trained ML model per Analytics ID (s) as indicated by the NWDAF containing AnLF.

The NWDAF containing AnLF subscribes to a (set of) trained ML Model(s) associated with a (set of) Analytics ID(s) by invoking the Nnwdaf_MLModelProvision_Subscribe service operation. The NWDAF service consumer includes the ML model file serialization format supported as input in Nnwdaf_MLModelProvision_Subscribe service operation.

The NWDAF containing MTLF notifies the NWDAF service consumer with the trained ML Model Information (containing (set of) file address of the trained ML model) by invoking Nnwdaf_MLModelProvision_Notify service operation. The content of trained ML Model Information that can be provided by the NWDAF containing MTLF is specified in clause 6.2A.2 of TS 23.288 [5].

6.14	Solution #14: Enhance trained ML Model sharing via ML Model format

6.14.1	Description

This is a solution for the Key Issue#5: Enhance trained ML Model sharing.

6.14.1.1	General

As shown in Figure1, ML model(s) is trained and inferred based on some specific AI framework (e.g. TensorFlow, Pytorch, Caffe, etc.) and it cannot be understood and used by other AI frameworks because the properties of each AI framework (e.g. Network topology, Execution flow, Data format, Supporting language) are different.

Figure 6.14.1.1-1: Problem of ML model(s) sharing between different AI Frameworks

As shown in Figure 6.14.1.1-2, it is proposed to enhance ML model sharing between different AI Frameworks via ML model Format such as ONNX(Open Neuro Network Exchange), which is an open format built to represent machine learning models. ONNX defines a common set of operators - the building blocks of machine learning and deep learning models - and a common file format to enable AI developers to use models with a variety of frameworks, tools, runtimes, and compilers, for example:

1).	For a NWDAF (containing MTLF) with TensorFlow AI framework, it only needs to convert a TensorFlow specific ML Mode file to an open ONNX ML Model file, which can be consumed by any NWDAF(AnLF) who supports open ONNX ML Model.

2).	For a NWDAF (containing AnLF) who supports open ONNX ML Model, it interacts with the NWDAF (containing MTLF) to retrieve the open ONNX ML Model file, which will be converted by NWDAF (containing AnLF) to Caffe specific ML Model file.

Editor's note:	ONNX is just an example of format for ML model, and which open format for ML Model and the associated software support for the ML model is used is FFS.

Figure 6.14.1.1-2: Open ML model Format to support ML model sharing

NOTE:	In addition, MPEG and JVET is studying how to use AI/ML model for video coding, where AI/ML model sharing between different companies has been conducted for video coding evaluation and crosscheck purposes:

-	Observations from JVET shows that conversion to/from ONNX can be done from various framework.

-	It was agreed to use ONNX format for model description for companies' evaluation of model performance.

-	Please refer to https://jvet-experts.org/ for detailed information.

6.14.1.2	Registration and discovery of NWDAF (containing MTLF) with ML model Format Information

Figure 6.14.1.2-1: Registration and discovery for NWDAF(MTLF) with open ML model Format

1-3.	The NWDAF (containing MTLF) registers its NF profile (Address of NWDAF (containing MTLF), Supported Analytics ID, Supported ML Model Format Information, Supported AI Framework Information) into NRF.

The Supported ML Model Format Information indicates which ML Model Format is supported. For example, the trained ML Mode by the NWDAF (containing MTLF) can be converted to an ONNX ML Model file.

The Supported AI Framework Information indicates which AI Framework (e.g. Tensor Flow, Pythorch, Caffe) is used to train the ML Model.

Editor's note:	It is FFS how to protect the ML model privacy in the MTLF, if the AnLF and the MTLF belongs to different vendors and coordination with SA3 is needed.

Editor's note:	It is FFS on what model metadata should be included in the model sharing procedure.

4-6.	The NWDAF (containing AnLF) is to discover multiple NWDAFs (containing MTLF) via the NRF by invoking the Nnrf_NFDiscovery_Request (Analytics ID, ML Model Format Information, AI Framework Information) service operation.

The NRF notifies the NWDAF (containing AnLF) with one or more NWDAF (containing MTLF) instances.

7.	The NWDAF (containing AnLF) selects a NWDAF (containing MTLF) and then subscribes to the chosen NWDAF (containing MTLF1) a trained ML Model by invoking the Nnwdaf_MLModelProvision (Analytics ID, ML Model Format Information, AI Framework Information) service operation.

8.	The chosen NWDAF (containing MTLF1) notifies the NWDAF (containing AnLF) with the ML model information by invoking Nnwdaf_MLModelProvision_Notify service operation.

The ML model Information includes either an ML Model file (e.g. an open ONNX ML Model file) or a specific AI Framework ML Model file (e.g. a TensorFlow specific ML Model file), respectively.

6.14.3	Impacts on existing nodes and functionality

NWDAF(MTLF):

-	register “Supported ML Model Format Information, Supported AI Framework Information” in NF profile to NRF.

-	provide ML Model Information per Supported ML Model Format Information/Supported AI Framework Information as indicated in Nnwdaf_MLModelProvision.

NRF:

-	support NWDAF(MTLF) discovery procedure per Supported ML Model Format Information/Supported AI Framework Information.

6.15	Solution #15: ML model sharing with different AnLF providers

6.15.1	Description

In Rel-17, NWDAF decomposed into as described in TS 23.288 [5]:

-	Analytics logical function (AnLF): An NWDAF containing the Analytics logical function can perform inference, derive analytics information and expose analytics.

-	Model Training logical function (MTLF): An NWDAF containing the Model Training logical function, trains ML models and exposes new training services.

But, even it has decomposed into two functions, the MTLF provides the ML model to the AnLF of same provider as MTLF i.e. ML model will be share to the same vendor AnLF, but not other vendor AnLF, see the below note from clause 5.1 of TS 23.288 [5].

NOTE 3:	In this Release of the specification an NWDAF containing AnLF is locally configured with (a set of) IDs of NWDAFs containing MTLF and the Analytics ID(s) supported by each NWDAF containing MTLF to retrieve trained ML models. An NWDAF containing AnLF uses NWDAF discovery for NWDAF containing MTLF within the set of configured IDs of NWDAFs containing MTLF, if necessary. ML Model provisioning/sharing between multiple MTLFs is not supported in this Release of the specification.

Therefore, based on the KI#5, in this solution, it is assumed that based on the business logic of the MTLF, the MTLF will determine whether to expose the certain ML models to another provider's AnLF or not. For example, assume that MTLF have two set of certain models, model#1 is developed based on publicly available model and model#2 is develop based on business needs with high accuracy level. In this case, MTLF provider only ready to share the model#1, but not model#2.

Based on the above assumption, this solution proposed that MTLF includes the support of interoperable indicator in the NF profile for the NRF registration, and the consumer (AnLF) discovery and select the MTLF that interoperability for an analytic ID. AnLF sends the Nnwdaf_MLModelInfo_Request with an Interoperable Indicator which encodes e.g. the 3GPP-specific Vendor Id (which is similar to the ViD as described in clause 23.3.2.3.1 of TS 23.003 [10]) to the MTLF. Based on the interoperable indicator, the MTLF determine and select the model, and sends the model details to the AnLF. Based on the local policy, if a MTLF does not support the model or does not want to share it, the MTLF will send a rejection message, e.g. reject message with that sharing the model is not possible for the analysis ID.

Furthermore, this solution also proposed that MTLF provide the accuracy level and resource efficiency of the ML model to the AnLF. Similar to preferred level of accuracy of the analytics, the MTLF determine the accuracy for the trained model and required resource efficiency (e.g. required resource to run the model).

6.15.2	Procedures

Figure 6.15.2-1: Procedure for sharing of ML model

1.	The AnLF discovery and selects the MTLF by local information or via from NRF. Each MTFL NF profile include the Interoperable Support Indicator, which indicate that the MTFL supports the interoperable models. The MTLF registers its NF profile in the NRF as described in clause 5.2.7.2.2 of TS 23.502 [3]. During the discovery of MTLF via NRF, the NRF provide MTLF profiles to the AnLF, the AnLF selects the MTLF that support the interoperability.

NOTE 1:	It is possible that AnLF can explicitly query the NRF supporting the interoperable MTLF by providing an Interoperable Support Indicator in the discovery request.

Editor's note:	Whether and which information should be included in the discovery request to NRF to improve the efficiency for MTLF discovery is FFS.

2.	The AnLF sends the Nnwdaf_MLModelInfo_Request to the MTLF with Interoperable Indicator (e.g. which encodes the 3GPP-specific Vendor Id) with an analytic ID and ML model filter information.

NOTE 2:	It is up to stage 3 to define how to encode the indicator with vendor ID. For example, Vendor ID data type as defined in TS 29.510 [11] or completely new format.

3.	MTLF determine the AnLF service provider and determine whether to expose the model or not. If yes, the MTLF selects the ML Model.

4.	The MTLF sends the ML models details to the AnLF in the Nnwdaf_MLModelInfo_Request response. In the response, the MTLF includes the ML model accuracy level and resource efficiency level to the AnLF.

NOTE 3:	It could be that AnLF can explicitly ask preferred level of ML model accuracy and resource efficiency in step 2. The accuracy and resource efficiency level can be "low", "medium" and "high".

6.15.3	Impacts on existing nodes and functionality

AnLF able to select the MTLF which support the interoperability. Furthermore, includes the Interoperable Indicator which encodes the 3GPP-specific Vendor Id in the Nnwdaf_MLModelInfo_Request and NRF service.

MTLF registers the support of interoperability with the NRF such as Interoperable Support Indicator in the NF profile and determine the AnLF vendor information. Furthermore, MTLF can provide the ML model accuracy and resource efficiency levels. Based on internal policy, the MTLF select the model and provide the AnLF.

NRF stores and filter MTLFs that support interoperability and provide the filtered MTLF to the AnLF.

6.16	Solution #16: NWDAF assisted URSP determination

6.16.1	Description

This solution is proposed to address Key Issue #6: NWDAF-assisted URSP.

A URSP rule consists of traffic descriptor and route selection descriptor. Traffic descriptor is determined e.g. by application guidance from AF and is relatively fixed, which may not need analytics from NWDAF. This solution mainly proposes the existing analytics and enhanced analytics from NWDAF to assist to adjust the route selection descriptor and the related components of route selection descriptors can be reflected by Analytics ID(s). And the mapping between RSD components and output from Analytics IDs are showed in Table 6.16.1-1.

Table 6.16.1-1: NWDAF assisted URSP determination

| Route selection components | Analytics ID(s) |
| --- | --- |
| Network Slice Selection | "Service Experience", "Load level information", "Dispersion Analytics", “Session Management Congestion Control Experience” |
| DNN Selection | "Service Experience", "UE Communication", “Session Management Congestion Control Experience” |
| Non-Seamless Offload indication | "UE Communication", "WLAN performance", "Load level information", "Network Performance", "User Data Congestion" |
| SSC Mode Selection | "Service Experience"; Enhancement is needed |
| PDU Session Type | "Service Experience"; Enhancement is needed |
| Access Type preference | "Service Experience"; Enhancement is needed |
| PDU Session Pair ID | “Redundant Transmission Experience” |
| RSN | “Redundant Transmission Experience” |
| Route selection validation criteria | Analytics ID(s) |
| Time Window | Based on the validity period and spatial validity provided in the Analytics ID used for RSD generation. |
| Location Criteria | Based on the validity period and spatial validity provided in the Analytics ID used for RSD generation. |

Editor's note:	Other usages of NWDAF analytics for URSP is FFS.

The PCF generates a URSP Rule based on the Service Parameters provided by the AF, or alternatively locally configured in the PCF or both, then the PCF checks that the RSC of the URSP Rule complies with the UE context Policy control subscription information for a SUPI. Then, the PCF checks if Analytics are needed for further refinement of the candidate values.

In this solution, the PCF is configured, depending on operator policies, to subscribe to those Analytics ID(s) listed in the Table 6.16.1-1, then output of these Analytics are used to:

-	determine if traffic of the application is to be offloaded outside the PDU Session.

-	prioritize (or remove some of) the list of candidate values for the Network Slice Selection component or for the DNN Selection component.

-	allocate the SSC Mode value, and the PDU Session Type.

-	select the preferred Access Type.

-	determine if PDU Session Pair ID and/or RSN needs to be provided.

These are some examples of how the PCF assigns the values of the different Route Selection Components:

Network Slice Selection: PCF can request or subscribe to:

-	Analytics ID "Service Experience" for a particular application that is part of the traffic descriptor, via the candidate network slices by setting filter information. The PCF may select or prioritize the network slice that provides the best service experience for the application.

-	Analytics ID "Load Level Information" for the candidate network slices and select the one with lowest load.

-	Analytics ID "Dispersion Analytics" to get, e.g. Data volume dispersion Analytics type. The PCF may calculate the average data rate in the network slice by subscribing to notifications of network analytics related to Data Volume Dispersion in the network slice for a duration of interest when it sets the Target of Analytics Reporting as "any UE". The PCF may update URSP on slice selection policy for the associated UEs based on the data volume dispersion analytics results provided by the NWDAF for data volume dispersion statistics at a given slice and/or data volume dispersion predictions at a given slice.

-	Analytics ID "Session Management Congestion Control Experience" for the PDU Session associated with the candidate S-NSSAI indicated by Analytics Filter Information, targeting the SUPI as Target of Analytics Reporting. The PCF may select or prioritize the S-NSSAI that provides the lowest experience level of Session Management Congestion Control for the SUPI.

-	Both, first the list of candidate slices and the service experience it provides for the application is provided and then the load level for those slices are checked.

-	Or just subscribe to Analytics ID "Service Experience" for a particular application, for slices that are not heavy loaded.

-	Or just to subscribe to Analytics ID "Dispersion Analytics" as explained above for certain slice(s).

DNN Selection: Similar to the first bullet, The PCF may subscribe to Analytics ID on "Service Experience" for a particular application that is part of the traffic descriptor, via the candidate DNNs. The PCF may select or prioritize some of the DNN that provides the best service experience for the application.

The PCF may also subscribe to notifications of network analytics related to "UE Communication". The PCF may update the URSP on DNN selection policy for the associated SUPI, group of UEs or UEs associated with the Application Identifier based on this notification from NWDAF (e.g. considering DNN in Traffic characterization and associated Traffic Volume, Spatial validity and inactivity time) . The PCF may subscribe to Analytics ID "Session Management Congestion Control Experience" for the PDU Session associated with the candidate DNN indicated by Analytics Filter Information, targeting the SUPI as Target of Analytics Reporting. The PCF may select or prioritize the DNN that provides the lowest experience level of Session Management Congestion Control for the SUPI.

Non-Seamless Offload indication: PCF can request or subscribe "UE Communication" to get information on the location where UE will use the application and then get the information about the WLAN performance of this location by requesting or subscribing the Analytics ID "WLAN performance". If the WLAN performance is good enough, PCF may choose to configure this indication for the application in that area to reduce the load of 3GPP access. PCF may subscribe to notifications of network analytics related to "Network Performance". The PCF may update this indication within URSP of the associated UEs within the area of interest based on this notification from NWDAF (e.g. on gNB status and gnB resource usage information). PCF may subscribe to notifications of network analytics related to "User Data Congestion". The PCF may update this indication within URSP of the associated SUPI based on this notification from NWDAF (e.g. Network Status Indication).

SSC Mode Selection, PDU Session Type and Access Type preference: the PCF may subscribe to Analytics ID on "Service Experience" for a particular application that is part of the traffic descriptor, via the candidate SSC mode, PDU Session Type and Access Type preference. The PCF may select the SSC mode, PDU Session Type and Access Type preference that provides the best service experience for the application. these factors should also be captured by "Service Experience" Analytics. This solution enhanced the "Service Experience" Analytics in clause 6.16.1.1 and 6.16.1.2. SSC Mode Selection: the PCF may subscribe to Analytics ID on "Expected UE behaviour" for this SUPI. The PCF may select the SSC mode depending on the mobility pattern (e.g. If no mobility then the SSC mode is not relevant and can be omitted).

PDU Session Pair ID and RSN: PCF can request or subscribe to Analytics ID "Redundant Transmission Experience" to be notified if PDU Session Pair ID and/or RSN needs to be provided. If the output of the Analytics says “redundant transmission shall be performed”, the PCF may provide appropriate PDU Session Pair ID and/or RSN with the help of local configuration in the corresponding SMF. Details on how the SMF determines PDU Session Pair ID and/or RSN can be found in clause 5.33.2.1 of TS 23.501 [2].

Time Window and Location Criteria: The Time Window in the RSD Validation Criteria can be set based on the time validity period and spatial validity in the Analytics ID(s) used as input in the RSD generation. The PCF may need to select the validity period and spatial validity out of those provided in the output of each Analytics ID(s).

6.16.1.1	Input Data

Based on input data of Observed Service Experience related network data Analytics as described in clause 6.4.2 of TS 23.288 [5], table 6.16.1.1-1 provides enhanced input data by adding PDU Session related information and table 6.16.1.1-2 provides enhanced input data access type.

Table 6.16.1.1-1: QoS flow level Network Data from 5GC NF related to the QoS profile assigned for a particular service (identified by an Application Id or IP filter information)

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | 5GC NF | A time stamp associated with the collected information. |
| Location | AMF | The UE location information, e.g. cell ID or TAI. |
| UE ID | AMF | (list of) SUPI(s). If UE IDs are not provided as Target of Analytics Reporting for slice service experience, AMF returns the UE IDs matching the AMF event filters. |
| DNN | SMF | DNN for the PDU Session which contains the QoS flow. |
| S-NSSAI | SMF | S-NSSAI for the PDU Session which contains the QoS flow. |
| Application ID | SMF | Used by NWDAF to identify the application service provider and application for the QoS flow. |
| UPF info | SMF | UPF ID/address/FQDN information for the UPF serving the UE. |
| DNAI | SMF | Identifies the access to DN to which the PDN session connects. |
| IP filter information | SMF | Provided by the SMF, which is used by NWDAF to identify the service data flow for policy control and/or differentiated charging for the QoS flow. |
| QFI | SMF | QoS Flow Identifier. |
| PDU Session type | SMF | Type of the PDU Session. |
| SSC Mode | SMF | SSC Mode selected for the PDU Session. |
| QoS flow Bit Rate | UPF | The observed bit rate for UL direction; and The observed bit rate for DL direction. |
| QoS flow Packet Delay | UPF | The observed Packet delay for UL direction; and The observed Packet delay for the DL direction. |
| Packet transmission | UPF | The observed number of packet transmission. |
| Packet retransmission | UPF | The observed number of packet retransmission. |

Table 6.16.1.1-1: UE level Network Data from 5G NF related to the Service Experience

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | 5GC NF | A time stamp associated with the collected information. |
| Location | AMF | The UE location information, e.g. cell ID or TAI. |
| UE ID | AMF | (list of) SUPI(s). |
| Access Type | AMF | The Access type the UE camps on. |
| RAT Type | SMF | The RAT type the UE camps on. |

Editor's note:	Whether and how to collect data from 4G access and non-3GPP access is FFS.

6.16.1.2	Output Analytics

Based on output analytics of Observed Service Experience related network data Analytics for applications as described in clause 6.4.3 of TS 23.288 [5], table 6.16.1.2-1 provides enhanced output analytics by adding PDU Session related information and access type.

Table 6.16.1.2-1: Service Experience Statistics

| Application service experiences (0..max) (NOTE 1) | List of observed service experience information for each Application. |
| --- | --- |
| > S-NSSAI | Identifies the Network Slice used to access the Application. |
| > Application ID | Identification of the Application. |
| > Service Experience Type | Type of Service Experience analytics, e.g. on voice, video, other. |
| > UE location | Indicating the UE location information (e.g. TAI list, gNB ID, etc) when the UE service is delivered (see NOTE 4). |
| > UPF Info | Indicating UPF serving the UE. |
| > DNAI | Indicating which DNAI the UE service uses/camps on. |
| > DNN | DNN for the PDU Session which contains the QoS flow. |
| > Application Server Instance Address | Identifies the Application Server Instance (IP address of the Application Server) or FQDN of Application Server. |
| > Service Experience | Service Experience over the Analytics target period (average, variance). |
| > SUPI list (0..SUPImax) | List of SUPI(s) with the same application service experience. |
| > Ratio | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity | Area where the Application service experience analytics applies. |
| > Validity period | Validity period for the Application service experience analytics as defined in clause 6.1.3. |
| > RAT Type | Indicating the RAT type for which the application service experience analytics applies. |
| > Frequency | Indicating the carrier frequency of UE's serving cell(s) where the application service experience analytics applies. |
| > SSC Mode | SSC Mode selected for the PDU Session used to associate with the application. |
| > PDU Session Type | Type of PDU Session used to associate with the application. |
| > Access Type | Access Type when the UE establishes a PDU Session for the application. |

6.16.2	Procedures

The same as procedures of related Analytics ID in TS 23.288 [5] and also clause 4.15.6.10 of TS 23.502 [3] with additional subscription (or request) to the NWDAF to determine the guidance for URSP.

6.16.3	Impacts on existing nodes and functionality

PCF:

-	Subscribes or requests Analytics ID (s) on Table 6.16.1-1 for URSP determination.

AF:

-	Subscribes or requests Analytics ID (s) on Table 6.16.1-1 to provide the guidance for URSP determination as described in clause 4.15.6.10 of TS 23.502 [3].

NWDAF:

-	supports enhanced Analytics ID "Service Experience".

Editor's note:	Other usages of NWDAF analytics for URSP is FFS.

SMF:

-	Nsmf_EventExposure, PDU Session Establishment and/or PDU Session Release is extended to include the SSC mode per PDU Session.

6.17	Solution #17: NSSP in roaming scenarios using network analytics

6.17.1	Description

This solution addresses key issue #3 "Data and analytics exchange in roaming case" and key issue #6 "NWDAF-assisted URSP".

When the UE is roaming, the PCF in the HPLMN may update the NSSP as part of the URSP rules in the UE. To decide the NSSP in roaming scenarios, the H-PCF may take into account of the Network Slice status in VPLMN. Therefore, analytics in VPLMN (i.e. Service Experience for an application and/or Network Slice, Network Slice load level, etc.) can be sent to the HPLMN and used by the H-PCF for decision of NSSP to be provisioned to the UE, so that the UE can be instructed to transmit the application data over the PDU Session(s) in the indicated VPLMN and/or HPLMN Network Slice(s) in roaming scenarios.

NOTE:	FS_UEPO, key issue 1, aims to enable the provisioning of URSP rules in the VPLMN and may thus remove the need for the H-PCF to determine NSSP.

6.17.2	Procedures

Figure 6.17.2-1 shows the procedure that H-NWDAF requests the Observed Service Experience analytics in VPLMN from V-NWDAF, upon receiving an analytics information/subscription request from the H-PCF.

Figure 6.17.2-1: Procedure for providing VPLMN Service Experience analytics to HPLMN

1.	H-PCF sends an Analytics request/subscribe (Analytics ID = Service Experience, Target of Analytics Reporting = a UE or a group of UEs or any UE, Analytics Filter Information = (Application ID, HPLMN S-NSSAI, VPLMN S-NSSAI, VPLMN ID)) to H-NWDAF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe.

NOTE:	Here the VPLMN S-NSSAI is used as an indication of request/subscription to analytics information of the mapped Network Slice in the VPLMN, and its value can be set to zero by the H-PCF.

2.	The H-NWDAF determines the VPLMN S-NSSAI, which is mapped to the HPLMN S-NSSAI, and discovers the V-NWDAF. The H-NWDAF invokes the Nnssf_NSSelection_Get service operation to the hNSSF with the HPLMN S-NSSAI and VPLMN ID received in step 1, and receives the VPLMN S-NSSAI and the information of appropriate vNRF to be used to select NFs/services within VPLMN, which may be obtained by the hNSSF from the vNSSF, in the Nnssf_NSSelection_Get response. The H-NWDAF further invokes the Nnrf_NFDiscovery_Request (Expected Service Name = Nnwdaf_AnalyticsInfo (or Nnwdaf_AnalyticsSubscription), NF type of the expected NF = NWDAF, Analytics ID = Service Experience, HPLMN ID, VPLMN ID, VPLMN S-NSSAI, HPLMN S-NSSAI, NF type of the NF service consumer = NWDAF) to the hNRF, and discovers the V-NWDAF using the procedure as described in clause 4.17.5 of TS 23.502 [3].

H-NWDAF sends an Analytics request/subscribe (Analytics ID = Service Experience, Analytics Filter Information = (Application ID, VPLMN S-NSSAI)) to V-NWDAF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe, based on the Analytics request/subscribe received from the H-PCF.

3-5.	V-NWDAF collects data from the NF(s) and/or OAM in VPLMN and derives the requested Service Experience analytics information, as specified in clause 6.4 of TS 23.288 [5].

6.	V-NWDAF sends the VPLMN Service Experience analytics information to the H-NWDAF using either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify.

7-9.	H-NWDAF collects data from the NF(s) and/or OAM in HPLMN and derives the requested Service Experience analytics information, as specified in clause 6.4 of TS 23.288 [5]. These steps can be executed in parallel with step 3-6.

10.	H-NWDAF sends the VPLMN analytics information received in step 6, together with the HPLMN analytics information derived in step 9, to the H-PCF using either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify.

When the UE is roaming, the PCF in the HPLMN may take into account of the VPLMN Service Experience analytics information to update the NSSP (as part of the URSP rules) in the UE, i.e. including the VPLMN S-NSSAI and/or HPLMN S-NSSAI of the Network Slice(s) with better Service Experience for the application, which instructs the UE to use the PDU Session in the Network Slice(s) for transmission of the application.

6.17.3	Impacts on existing nodes and functionality

PCF: based on HPLMN and VPLMN analytics information, updates the NSSP (as part of the URSP rules) to the UE for the UE using the PDU Session in the indicated Network Slice(s) to transmit the application in roaming scenarios.

H-NWDAF: requests VPLMN analytics information from the V-NWDAF.

V-NWDAF: provides VPLMN analytics information to the H-NWDAF.

6.18	Solution #18: Integrity KPI for QoS Sustainability Analytics

6.18.1	Key Issue mapping

This key issue addresses Key Issue#7 i.e. enhancements on QoS sustainability analytics.

6.18.2	Description

In general, this solution proposes to use QoS sustainability analytics solution which is defined in clause 6.9 of TS 23.288 [5] as baseline and add new input and output parameters to meet 5GAA requirement. It is assumed to reuse the same Analytics ID as defined Rel-17.

The consumer of QoS Sustainability analytics may request the NWDAF analytics information regarding the QoS change statistics for an Analytics target period in the past in a certain area or the likelihood of a QoS change for an analytics target period in the future in a certain area with finer granularity i.e. below cell level.

The consumer can request either to subscribe to notifications (i.e. a Subscribe-Notify model) or to a single notification (i.e. a Request-Response model).

The service consumer may be a NF (e.g. AF).

The request includes the following parameters:

-	Analytics ID = "QoS Sustainability ";

-	Target of Analytics Reporting: any UE;

-	Analytics Filter Information:

-	QoS requirements (mandatory):

-	5QI (standardized or pre-configured), and applicable additional QoS parameters and the corresponding values (conditional, i.e. it is needed for GBR 5QIs to know the GFBR); or

-	the QoS Characteristics attributes including Resource Type, PDB, PER and their values;

-	Location information (mandatory): an area or a path of interest. The location information could reflect a list of waypoints;

NOTE:	In this Release, the consumer of the "QoS Sustainability" Analytics ID will provide location information in the area of interest format (TAIs or Cell IDs) and also location information below cell level which are understandable by NWDAF.

-	S-NSSAI (optional);

-	Optional maximum number of objects;

-	Analytics target period: relative time interval, either in the past or in the future, that indicates the time period for which the QoS Sustainability analytics is requested;

-	Reporting Threshold(s), which apply only for subscriptions and indicate conditions on the level to be reached for the reporting of the analytics, i.e. to discretize the output analytics and to trigger the notification when the threshold(s) provided in the analytics subscription are crossed by the expected QoS KPIs.

-	A matching direction may be provided such as crossed (default value), below, or above.

-	An acceptable deviation from the threshold level in the non-critical direction (i.e. in which the QoS is improving) may be set to limit the amount of signalling.

The level(s) relate to value(s) of the QoS KPIs defined in TS 28.554 [9], for the relevant 5QI:

-	for a 5QI of GBR resource type, the Reporting Threshold(s) refer to the QoS flow Retainability KPI;

-	for a 5QI of non-GBR resource type, the Reporting Threshold(s) refer to the RAN UE Throughput KPI.

-	for any 5QI, the Reporting Threshold(s) refer to the Integrity KPI which contains latency and delay of 5G networks as defined in TS 28.554 [9].

-	In a subscription, the Notification Correlation Id and the Notification Target Address.

The NWDAF collects the corresponding statistics information on the QoS and Integrity KPI for the relevant 5QI of interests from the OAM, i.e. the QoS flow retainability or the RAN UE Throughput as defined in TS 28.554 [9].

If the Analytics target period refers to the past:

-	The NWDAF verifies whether the triggering conditions for the notification of QoS change statistics are met and if so, generates for the consumer one or more notifications.

-	The analytics feedback contains the information on the location and the time for the QoS change statistics and the Reporting Threshold(s) that were crossed.

If the Analytics target period is in the future:

-	The NWDAF detects the need for notification about a potential QoS change based on comparing the expected values for the KPI of the target 5QI against the Reporting Threshold(s) provided by the consumer in any cell in the requested area for the requested Analytics target period. The expected KPI values are derived from the statistics for the 5QI obtained from OAM. OAM information may also include planned or unplanned outages detection and other information that is not in scope for 3GPP to discuss in detail.

-	The analytics feedback contains the information on the location and the time when a potential QoS change may occur and what Reporting Threshold(s) may be crossed.

6.18.3	Procedures

Figure 6.18.3-1 depicts a procedure for QoS Sustainability analytics provided by NWDAF.

Figure 6.18.3-1: QoS Sustainability analytics provided by NWDAF

1.	The consumer requests or subscribes to analytics information on "QoS Sustainability" provided by NWDAF. The parameters included in the request are described in clause 6.9.1 of TS 23.288 [5].

The consumer may include multiple sets of parameters in order to provide different combinations of "Location information" and "Analytics target period" when requesting QoS Sustainability analytics.

2.	The NWDAF collects the data specified in clause 6.9.2 of TS 23.288 [5] from the OAM, following the procedure captured in clause 6.2.3.2 of TS 23.288 [5].

3.	The NWDAF verifies whether the triggering conditions are met and derives the requested analytics. The NWDAF can detect the need for notification based on comparing the requested analytics of the target 5QI against the Reporting Threshold(s) provided by consumer in any cell over the requested Analytics target period.

4.	The NWDAF provides response or notification on "QoS Sustainability" to the consumer.

6.18.4	Impacts on services, entities and interfaces

6.18.4.1	Input data

Table 6.18.4.1-1: Data collection for "QoS Sustainability" analytics

| Information | Source | Description |
| --- | --- | --- |
| RAN UE Throughput | OAM TS 28.554 [9] | Average UE bitrate in the cell (Payload data volume on RLC level per elapsed time unit on the air interface, for transfers restricted by the air interface), per timeslot, per cell, per 5QI and per S-NSSAI. |
| QoS flow Retainability | OAM TS 28.554 [9] | Number of abnormally released QoS flows during the time the QoS Flows were used per timeslot, per cell, per 5QI and per S-NSSAI. |
| Latency and delay of 5G networks | OAM TS 28.554 [9] | Uplink and downlink delay of 5G network |

NOTE:	The timeslot is the time interval split according to the time unit of the OAM statistics defined by operator.

Editor's note:	How to collect the finer granularity about location is based on the output in KI#9.

6.18.4.2	Output analytics

The NWDAF outputs the QoS Sustainability analytics. Depending on the Analytics target period, the output consists of statistics or predictions. The detailed information provided by the NWDAF is defined in Table 6.9.3-1 for statistics and Table 6.9.3-2 for predictions.

Table 6.9.3-1: "QoS Sustainability" statistics, TS 23.288 [5]

| Information | Description |
| --- | --- |
| List of QoS sustainability Analytics (1..max) |  |
| >Applicable Area with finer granularity | A list of TAIs or Cell IDs as well as below cell level location information that the analytics applies to. |
| >Applicable Time Period | The time period within the Analytics target period that the analytics applies to. |
| >Crossed Reporting Threshold(s) | The Reporting Threshold(s) that are met or exceeded or crossed by the statistics value or the expected value of the QoS and Integrity KPI. |

Table 6.9.3-2: "QoS Sustainability" predictions, TS 23.288 [5]

| Information | Description |
| --- | --- |
| List of QoS sustainability Analytics (1..max) |  |
| >Applicable Area with finer granularity | A list of TAIs or Cell IDs as well as below cell level location information that the analytics applies to. |
| >Applicable Time Period | The time period within the Analytics target period that the analytics applies to. |
| >Crossed Reporting Threshold(s) | The Reporting Threshold(s) that are met or exceeded or crossed by the statistics value or the expected value of the QoS and Integrity KPI. |
| >Confidence | Confidence of the prediction. |

NOTE 1:	The meaning of Confidence is based on the SLA, i.e. the consumer has to understand the meaning of the different values of Confidence.

NOTE 2:	The Analytics can contain multiple sets of the above information if the location information reflected a list of waypoints.

The number of QoS sustainability analytics entries is limited by the maximum number of objects provided as part of Analytics Reporting Information.

6.19	Solution #19: Enhanced QoS Sustainability Analytics in the finer granularity area

6.19.1	Description

QoS Sustainability analytics is an important analytics service, especially for V2X-related or safety related applications, since it helps to handle the impact of a sudden change of the QoS and thus avoid a harsh application adjustment, which could affect safety and efficiency. However, there is the need to enhance the granularity and the accuracy of the QoS Sustainability analytics as well as to extend the QoS parameters that the QoS Sustainability notification can be provided. The latter will help to increase the usefulness of the specific analytics ID as well as to support the requirements of various use cases (e.g. V2X related).

More accurate QoS Sustainability outputs needs QoS and input parameters to be associated with more accurate location information, when input information is monitored at RAN. Also, the list of input features of the QoS Sustainability analytics ID should be extended by adding information on radio resource utilization and performance measurements for transfer over the UP to improve the correctness and accuracy of QoS sustainability output.

Additional QoS parameters (delay, packet loss rate) should be considered by the QoS Sustainability analytics and additional inputs for these QoS parameters are needed as inputs to the QoS Sustainability analytics. Finally, in the case that an area of interest is provided in the QoS Sustainability request message by an analytics consumer then the necessary granularity of the area of interest should be indicated.

The outputs of the QoS Sustainability analytics (statistics and predictions), should be provided for more granular location, according to the request with an area or a path of interest.

6.19.2	Procedures

6.19.2.1	General

According to the description in clause 6.9 in TS 23.288 [5] the consumer of QoS Sustainability analytics may request the NWDAF analytics information regarding the QoS change statistics for an Analytics target period in the past in a certain area or the likelihood of a QoS change for an Analytics target period in the future in a certain area or path of interest, which is smaller than a cell.

Editor's note:	If the AF cannot provide the certain area or path of interest, which is smaller than cell, it is FFS how the NWDAF determines an area smaller than cell for the QoS sustainability analytics.

The area is described in the request message (either of a Subscribe-Notify model or of a Request-Response model) via Location information (mandatory):

-	a path of interest: The location information could reflect a list of waypoints;

-	an area: The location information could reflect expected granularity levels of the requested area.

6.19.2.2	Input data

The table with input data of the QoS Sustainability analytics (TS 23.288 [5]) should be updated by adding information on radio resource utilization and input related to other QoS parameters (e.g. delay, packet error rate).

Table 6.19.2.2-1: Data collection for "QoS Sustainability" analytics (TS 23.288 [5])

| Information | Source | Description |
| --- | --- | --- |
| RAN UE Throughput | OAM TS 28.554 [9] | Average UE bitrate in the Area or path of Interest (Payload data volume on RLC level per elapsed time unit on the air interface, for transfers restricted by the air interface), per timeslot, per cell, per 5QI and per S-NSSAI. |
| QoS flow Retainability | OAM TS 28.554 [9] | Number of abnormally released QoS flows during the time the QoS Flows were used per timeslot, per cell, per 5QI and per S-NSSAI. |
| RAN status, load and performance information | OAM | Statistics on RAN status (up/down), load (i.e. Radio Resource Utilization) and performance in the Area or path of Interest as defined in TS 28.552 [7] |
| Packet Delay | OAM | Statistics on RAN delay measurements (e.g. Average delay UL/DL air-interface), Round-trip GTP Data Packet Delay, packet delay between NG-RAN and PSA UPF in the Area or path of Interest as defined in TS 28.552 [7] |
| Packet Loss and/or Drop | OAM | packet loss (UL/DL) information in the NG-RAN in the Area or path of Interest as defined in TS 28.552 [7] |

NOTE 1:	The timeslot is the time interval split according to the time unit of the OAM statistics defined by operator.

NOTE 2:	Whether the information listed in Table 6.19.2.2-1 can be collected in requested Area or path of Interest from AF should be dependent on RAN WG3/SA WG5.

With the OAM data per finer granularity area in Table 6.19.2.2-1, the NWDAF derives the "QoS Sustainability" statistics on the finer granularity area.

Alternatively, if the information listed in Table 6.19.2.2-1 can be collected from the OAM in requested Area or path of Interest, additional input data for the QoS Sustainability analytics is shown in Table 6.19.2.2-2.

Note that the NWDAF should firstly determine the Cell list or TA list where requested area or path of Interest is included before the data collection from LMF, AMF, SMF and PCF.

Editor's note:	How NWDAF gets the UE list located in the finer granularity area is FFS.

Table 6.19.2.2-2: Additional data collection for "QoS Sustainability" analytics

| Information | Source | Description |
| --- | --- | --- |
| Timestamp |  | A time stamp associated with the collected information. |
| UE ID | LMF, AMF | SUPI |
| UE locations (1..max) | LMF, AMF | UE positions located in the Cell or TA where requested area or path of Interest is included |
| >UE location |  | Geodetic Location that the UE is located |
| >Timestamp |  | A time stamp when the LMF calculates the UE is located in this location |
| QoS Notification Control of the QoS profiles or Alternative QoS Profiles | SMF/PCF | QNC or AQP events from SMF/PCF to indicate that the GFBR, the PDB or the PER of the QoS profile cannot be fulfilled or can be fulfilled again. |
| QoS profiles | SMF/PCF | QoS profiles corresponding to the QNC or AQP |

NOTE:	The new input about QoS profile from SMF/PCF corresponding to the QNC or AQP will not trigger new RAN measurement.

Note that based on the current QNC/AQP notification mechanism, the SMF/PCF cannot know which parameter of the PER, PDB or GFBR cannot be fulfilled from the QNC/AQP="GFBR can no longer be guaranteed" event. On one hand, because the V2X Application based on the QoS sustainability analytics is quite safety sensitive, when QNC/AQP="GFBR can no longer be guaranteed", the NWDAF can default assume that all the GFBR/PDB/PER are not fulfilled in RAN and send the analytics to V2X AF to help application adjustment. Alternatively, the NWDAF can try to figure the reason out on its own by comparing the RAN notification (i.e. a new reference for the newly fulfilled QoS profile) with the previously fulfilled QoS profile, i.e. if only one of the values has changed the reason is that parameter (if two or three have changed, the NWDAF does not know which parameter(s) caused the problem).

This solution does not provide any enhancement on the current QNC/QAP triggering mechanism i.e. NWDAF can only collect the current QNC/AQP events triggered by the AF. It means that the signalling pressure is not too high since signalling is not collected for every QoS Sustainability request, Notification control is enabled when the QoS Notification Control parameter is set in the PCC rule (received from the PCF) that is bound to the QoS Flow.

NWDAF subscribes to the network data from AMF in the Table 6.19.2.2-2 by invoking Namf_EventExposure_Subscribe (Event IDs = Location Changes, Area of Interest=Cell list or TA list).

Editor's note:	How to collect the UE location from LMF is FFS and the cooperation with FS_eLCS_Ph3 is needed.

NWDAF subscribes the network data from SMF/PCF in the Table 6.19.2.2-2 by using the services provided by SMF/PCF together with the Cell list or TA list.

Editor's note:	Whether the network data collection from data sources should be considered except from SMF/PCF, e.g. UPF, OAM(MDT), is FFS.

The correlation information in clause 6.2.4 of TS 23.288 [5] are re-used to correlate the data from LMF, AMF and SMF/PCF.

With the CN data per UE in the finer granularity area in Table 6.19.2.2-2, the NWDAF derives the "QoS Sustainability" statistics on the finer granularity area. For example, the NWDAF can derives the analytics as follows:

-	how many percent of the UE's bit rate/packet delay/packet error rate which can be or cannot be guaranteed on the finer granularity area (below cell level); or

-	Average bit rate/packet delay/packet error rate can be or cannot be guaranteed on the finer granularity area (below cell level).

6.19.2.3	Output analytics

The NWDAF outputs the QoS Sustainability analytics, which should be provided according to the analytics request area of path of interest granularity. The provided granularity depends also on collected monitoring data (e.g. that the model has been trained).

Also, the Crossed Reporting Threshold(s) should be indicated for QoS parameter: Throughput, Delay, Packet loss.

Table 6.19.2.3-1: "QoS Sustainability" statistics

| Information | Description |
| --- | --- |
| List of QoS sustainability Analytics (1..max) |  |
| >Applicable Area | the Location information that the analytics applies to, the Area or path of Interest according to the analytics request |
| >Applicable Time Period | The time period within the Analytics target period that the analytics applies to. |
| >Crossed Reporting Threshold(s) | The Reporting Threshold(s) for each QoS parameter that are met or exceeded or crossed by the statistics value or the expected value of the QoS KPI. |

Table 6.19.2.3-2: "QoS Sustainability" predictions

| Information | Description |
| --- | --- |
| List of QoS sustainability Analytics (1..max) |  |
| >Applicable Area | the Location information that the analytics applies to, the Area or path of Interest according to the analytics request |
| >Applicable Time Period | The time period within the Analytics target period that the analytics applies to. |
| >Crossed Reporting Threshold(s) | The Reporting Threshold(s) for each QoS parameter that are met or exceeded or crossed by the statistics value or the expected value of the QoS KPI. |
| >Confidence | Confidence of the prediction. |

6.19.2.4	Procedures

The procedure in Figure 6.9.4-1 of TS 23.288 [5] is re-used with additional data collection step from LMF, AMF, etc.

Editor's note:	The procedure will be added when the service operation for LMF data collection is defined.

6.19.3	Impacts on existing nodes and functionality

NWDAF:

-	Monitor more information by OAM including radio resource utilization and input related to other QoS parameters or collects QNC/AQP events from SMF/PCF, UE Location from LMF or AMF;

-	The outputs of the QoS Sustainability analytics (statistics and predictions), should be provide:

-	Crossed Reporting Threshold(s) for QoS parameter: Throughput, Delay, Packet loss.

-	more granular location, according to the request with an area or a path of interest.

Consumer NFs:

-	In the case that an area of interest is provided in the QoS Sustainability request message by an analytics consumer then the necessary granularity of the area of interest should be indicated.

6.20	Solution #20: GTP metrics for QoS Sustainability analytics

6.20.1	Description

This solution addresses the Key Issue #7: Enhancements on QoS Sustainability analytics.

In Rel-17, the NWDAF supporting QoS Sustainability analytics collects UE QoS flow Retainability KPI and RAN UE Throughput KPI from OAM (see clause 6.9.2 of TS 23.288 [5]).

This solution proposes that the NWDAF additionally collects the following input (see Table 6.20.2-1) according to existing measurements defined in clause 5.33.3 of TS 23.501 [2] (QoS Monitoring to Assist URLLC Service).

In order to extend to Network Performance measurements following IP-layer clause capacity and IP-layer available clause capacity definition from ITU-T Y.1540 [8], the NWDAF should also collect capacity metrics based on GTP path (see Table 6.20.2-2) between UPF, NG-RAN and UE.

NOTE:	How OAM can perform capacity measurements is to be determined by SA WG5.

6.20.2	Input data

Table 6.20.2-1: Packet drop and/or packet delay measurement at GTP level

| Information | Source | Description |
| --- | --- | --- |
| UL/DL packet delay GTP | SMF or UPF (NOTE 2) | End-to-End measurements from UE to UPF. |
| UL/DL packet delay GTP | OAM (NOTE 3) | UL/DL packet delay measurement round trip on GTP path on N3. |
| NOTE 2: Direct collection from UPF depends on the outcome of FS_UPEAS. NOTE 3: Refer to TS 28.552 [7] clause 5.1 for the performance measurement in NG-RAN and TS 28.552 [7] clause 5.4 for the performance measurement in UPF. In addition, Annex A of TS 28.552 [7] describes various performance measurements, especially, Annex A.61 "Monitoring of one way delay between PSA UPF and NG-RAN" indicates that the measurements on the one way DL and UL delay between PSA UPF and NG-RAN can be used to evaluate and optimize the DL and UL user plane delay performance between 5GC and NG-RAN. Whether the mentioned RAN measurements are needed or even available per UE will be looked into in normative phase. NOTE 4: The NWDAF actions will not activate the measurements for QoS Monitoring. When QoS Monitoring is activated, the measurements are triggered according to existing procedures. When QoS Monitoring is not activated SMF may per configuration activate measurements for QoS Monitoring at least at the same time as it requests Analytics. |  |  |

The following new data collection proposes to extend the measurements related to QoS Monitoring and defined in TS 23.501 [2] and TS 28.552 [7] to include the IP-layer section capacity and IP-layer available section capacity between UE, NG-RAN and UPF at GTP level.

Table 6.20.2-2: New Data collection for QoS Sustainability analytics

| Information | Source | Description |
| --- | --- | --- |
| UL/DL capacity GTP between UPF and NG-RAN | OAM (NOTE 3) TS 28.552 [7] | UL/DL capacity measurement from UPF to NG-RAN based on GTP path. The capacity measurement corresponds to the IP-layer section capacity definition from ITU-T Y.1540 [8]. |
| UL/DL capacity GTP between UPF and UE | OAM (NOTE 3) TS 28.552 [7] | UL/DL capacity measurement from UE to UPF based on GTP path. The capacity measurement corresponds to the IP-layer section capacity definition from ITU-T Y.1540 [8]. |
| UL/DL available capacity GTP between UPF and NG-RAN | OAM (NOTE 3) TS 28.552 [7] | UL/DL available capacity measurement from UPF to NG-RAN based on GTP path. The available capacity measurement corresponds to the IP-layer available section capacity definition from ITU-T Y.1540 [8]. |
| UL/DL available capacity GTP between UPF and UE | OAM (NOTE 3) TS 28.552 [7] | UL/DL available capacity measurement from UE to UPF based on GTP path. The available capacity measurement corresponds to the IP-layer available section capacity definition from ITU-T Y.1540 [8]. |
| NOTE 1: UL/DL packet latency is already available at GTP level. NOTE 2: NWDAF could also collect the information from UPF, depending on the outcome of FS_UPEAS. NOTE 3: New performance measurement needs to be defined by SA5 for capacity measurement in addition to clause 5.1 of TS 28.552 [7] for the performance measurement in NG-RAN and clause 5.4 1 of TS 28.552 [7] for the performance measurement in UPF. In addition, Annex A of TS 28.552 [7] describes various performance measurements. NOTE 4: The NWDAF data collection request/subscription will not activate the measurements for QoS Monitoring. When QoS Monitoring is activated, the measurements are triggered according to existing procedures. When QoS Monitoring is not activated, SMF may per configuration activate measurements for QoS Monitoring at least at the same time as it requests Analytics. |  |  |

6.20.3	Procedures

The procedure for "QoS Sustainability" analytics provided by NWDAF specified in clause 6.9.4 of TS 23.288 [5] is enhanced by adding data collection from SMF or UPF in step 2.

NOTE:	Direct data collection from UPF depends on the outcome of FS_UPEAS.

6.20.3	Impacts on services, existing nodes and functionality

NWDAF:

-	Consume UL/DL packet delay.

-	Consume UL/DL new capacity metrics between UPF and UE and between UPF and NG-RAN.

-	Consume UL/DL new available capacity metrics between UPF and UE and between UPF and NG-RAN.

UPF:

-	Perform the measurements described in clause 6.20.2.

6.21	Solution #21: Federated Learning procedure between different NWDAFs

6.21.1	Description

This solution is proposed for KI#8 to support the Federated Learning procedure between different NWDAFs containing MTLF.

6.21.2	Procedures

6.21.2.1	NWDAF "FL Capability" Registration

During the NWDAF containing MTLF registration in NRF procedure:

-	The NWDAF containing MTLF shall include the "FL capability" registered in its NF Profile registered in the NRF.

-	The "FL capability" indicates to support "FLaggregation" and / or "FL participant" for the corresponding Analytics ID.

-	"FLaggregation" capability means the MTLF is able to manage the FL operation, select FLparticipants, aggregate the ML models from different ML clients and send back the trained ML model information to the FL participants.

-	"FL participant" capability means the MTLF is able to perform the ML model training for the ML model that requested by the "FL aggregation" by using the available local database and report the trained ML model information to the "FL aggregation". The local database can be the data that is allowed to collect by the MTLF from other 5GS or ADRF.

6.21.2.2	NWDAF Selection for FL operation

“FL Consumer” NWDAF is either an NWDAF containing AnLF, or an NWDAF containing MTLF that does not support the “FL aggregation” capability for the specific Analytics ID, but it is able to enable FL for his/her ML model by requesting FL to “FL Server” NWDAF.

"FL Server" NWDAF is the NWDAF containing MTLF that supports the "FL aggregation" capability for the specific Analytics ID and selected as the FL Server.

"FL Client" NWDAF is the NWDAF containing MTLF that supports the "FL participant" capability for the specific Analytics ID and selected by the "FL Server" NWDAF as the FL Client.

The "FL Server" NWDAF may be selected based on operator's local configuration or by the NWDAF containing AnLF for the analytics ID.

When the “FL Consumer” NWDAF is an NWDAF containing AnLF and it discovers an MTLF for a trained ML model as described in clause 5.2 of TS 23.288 [5]. If there is no trained ML model for the Analytics ID supported, the NRF will select the NWDAF containing MTLFs that supports the Analytics ID and the Area Of Interest to train the ML model. The NRF may take priority to select the NWDAF(s) containing MTLF that support "FL aggregation" capability if multiple NWDAF containing MTLF profiles registered in the NRF can support the requested ML model training, NRF returns the candidate for instances of NWDAF containing MTLF sent to consumer. The NWDAF containing AnLF selects one NWDAF containing MTLF from the list that received from NRF and initiates the ML Model Provisioning procedure as described in clause 6.2A of TS 23.288 [5]. If the NWDAF containing MTLF selected by NWDAF containing AnLF supports the "FL aggregation" capability for the Analytics ID, the NWDAF containing MTLF will further decide if FL procedure is initiated. If the NWDAF containing MTLF selected by NWDAF containing AnLF does not support the "FL aggregation" capability, the NWDAF containing MTLF may either performs the ML model training and provision as defined in clause 6.2A in TS 23.288 [5] or discovers a NWDAF containing MTLF that supports “FL aggregation” capability to trigger FL operation.When “FL Consumer” NWDAF is an NWDAF containing MTLF and it determines to trigger FL by internal logic (e.g., lack of data to train, heavy overhead from the existing ML training, etc.), “FL Consumer” NWDAF discovers an NWDAF which supports “FL aggregation” capability via NRF and initiates FL by invoking a Nnwdaf_MLModelTraining_Subscribe operation with the indication of enabling FL and initial ML model file address to the selected NWDAF (i.e., “FL Sever” NWDAF).

The "FL Server" NWDAF selects the "FL Client" NWDAF from NRF as described in clause 5.2 of TS 23.288 [5]. The "FL Server" NWDAF further includes the "FL Client" capability as "FL participant" capability and the number of reported NWDAFs for the Analytics ID in the discovery request message. The NRF returns the candidates for instances of NWDAF containing MTLF and each candidate for instance of NWDAF containing MTLF includes the Analytics ID, the ML Model Filter Information and support of "FL participant" for the Analytics ID.

6.21.2.3	FL operation between NWDAFs containing MTLF

The FL operation procedure between NWDAFs containing MTLF is described in Figure 6.21.2.3-1.

Figure 6.21.2.3-1: FL operation procedure between NWDAFs

0a.	When “FL Consumer” NWDAF is an NWDAF containing MTLF without “FL aggregation” capability and it determines to trigger FL from the internal logic, the “FL Consumer” NWDAF discovers and selects “FL Server” NWDAF via NRF, and subscribes Nnwdaf_MLModelTraining_Subscribe service to the “FL Sever” NWDAF with indication of “Enabling FL” and the file address of ML model to train.

0b.	When “FL Consumer” NWDAF is an NWDAF containing AnLF, the “FL Consumer” NWDAF discovers and selects “FL” Server NWDAF via NRF, and subscribes Nnwdaf_MLModelProvsion_Subscribe service to the “FL Server” as described in clause 6.2A of TS 23.288 [5].

0c.	T	he FL operation maybe triggered by local configuration in the "FL Server" NWDAF itself or request from “FL Consumer” by either step 0a or step 0b. If the Nnwdaf_MLModelProvision_Subscribe operation is triggered at step 0b, the “FL Server” NWDAF will consider whether FL operation is needed or not by taking into account the Area Of Interest, the ML model Reporting Information, Expiry time and local operator configuration.

The "FL Server" NWDAF is configured the Initial FL parameters corresponding to the Analytics ID:

-	Number of FL rounds,

-	Total number of FL clients used in the process.

-	The area of interesting for the analytics ID, the area of interesting can be a PLMN or a registration area list.

For Initial ML model training, the pre-configured Initial FL parameters will be used.

The ML model maybe re-trained to improve correctness of NWDAF analytics as discussed in KI#1, the FL parameters used for re-training maybe different to the pre-configured Initial FL parameters.

NOTE 1:	The initiation of ML model re-training and the details of FL parameter should be aligned with the conclusion of KI#1.

1.	"FL Server" NWDAF selects the "FL Client" NWDAFs for the specific Analytics ID within the area of interesting for the FL as described in clause 6.21.2.2. "FL Server" NWDAF sends Nnwdaf_MLModelTraining_Subscribe to the "FL Client" NWDAFs with additional parameters:

-	Indication for “Enabling Local Training”

-	the FL ID: For each ML model FL operation, the "FL Server" NWDAF will allocate different FL IDs for different "FL Client" NWDAFs. The "FL Server" NWDAF will identify the received the interim ML models from different "FL Client" via Analytics ID + FL ID. The FL ID will not updated during the FL operation procedure.

-	ML model information (containing a (set of) file address of the trained ML model) to request the ML model operation.

NOTE 2:	The selection of FL Clients and the detail parameters to share the ML model should be aligned with the conclusion of KI #5.

2.	For each "FL Client" NWDAF, if the ML model information is received from consumer, the "FL Client" NWDAF downloads the ML model according to the ML model information received from step 1.

3.	User consent will be performed by each "FL Client" NWDAF if UE related parameters are required for ML training.

4.	Each "FL Client" NWDAF may collect data from other available 5GS (e.g. AMF, SMF) or DCCF for local ML model training.

5.	Data source sends data to "FL Client" NWDAF.

6.	Each "FL Client" NWDAF performs local ML model training and sends the trained interim ML model via Nnwdaf_MLModelTraining_Notify to the "FL Server" NWDAF. The Analytics ID, FL ID, File address of learning results and ML Model Filter Information for the trained interim ML model by "FL Client" NWDAF are included in the notify message.

7.	"FL Server" NWDAF downloads interim trained ML models Identified by Analytics ID and FL ID trained by each "FL Client" NWDAF according to the ML model information received in step 6.

8.	"FL Server" NWDAF performs aggregation for the ML models for the same analytics ID.

Step 2 to step 8 in box A may repeated to support iterative learning based on the FL parameters configured for the Analytics ID expect step 3. For example, after the step 8 for the first round of learning, the "FL Server" NWDAF will initiate step 1 for the second round of learning.

Use consent check is only performed for in the first round of iterative learning.

9-10.	The "FL Client" NWDAF may subscribe the trained ML model for the Analytics ID from "FL Server" NWDAF as described in clause 6.2A.1 of TS 23.288 [5].

11a.	“FL Sever” provides the trained ML models by FL to the “FL Consumer” NWDAF at step 0a by invoking “Nnwdaf_MLModelTraining_Notify” service operation.

11b.	“FL Sever” provides the trained ML models by FL to the “FL Consumer” NWDAF at step 0b by invoking “Nnwdaf_MLModelProvision_Notify” service operation.

NOTE 2:	The ML model provision between "FL Server" NWDAF and "FL Client" NWDAF in this solution should align with the conclusion in KI#5.

6.21.3	Impacts on existing nodes and functionality

NWDAF containing MTLF:

-	register the "FL capability" in NF profile to NRF.

For "FL Server" NWDAF:

-	support "FL Client" NWDAF discovery procedure.

-	support to send FL request to "FL Client" NWDAF.

-	support to aggregate the interim learning results from "FL Clients" NWDAF.

-	support a new service “Nnwdaf_MLModelTraining”.

For "FL Client" NWDAF:

-	support local ML training based on "FL Server" NWDAF request.

-	support to report the trained interim ML model to "FL Server" NWDAF.

-	support a new service “Nnwdaf_MLModelTraining”.

6.22	Solution #22: Federated learning group creation

6.22.1	Description

The solution addresses Key Issue #8: Supporting Federated Learning in 5GC with proposing a procedure to create Federated Learning group of NWDAFs for a given analytics request from any NF.

To enable Federated Learning among NWDAFs, it is required to support creation of a Federated Learning group for a given analytics request. In this regard, this solution focuses on how to discover proper participant NWDAFs supporting Federated Learning, create the Federation Learning group, and provision information required to coordinate the operation among the participant NWDAFs.

This solution is based on the following assumption:

-	A Federated Learning group consists of one central NWDAF and multiple local NWDAFs. The local NWDAF performs local ML model training and provide local ML model to the central NWDAF. The central NWDAF aggregates the local ML model trained by local NWDAFs in the Federated Learning group, calculate and distribute a global ML model to the local NWDAFs.

-	For a given Federated Learning group, the same ML model is used by all the participant NWDAF analytics.

6.22.2	Procedures

Figure 6.22.2-1: Federated learning group creation procedure

0.	NWDAF(s) respectively invokes an Nnrf_NFManagement_NFRegister Request service providing their own NF profile that includes supported Analytics ID, ML Model Filter Information (e.g. S-NSSAI(s), Area(s) of Interest) and ML model sharing capability, which is required to send locally trained model to the Central NWDAF during federation learning operations as local NWDAF, when becoming operative for the first time.

Editor's note:	The aspects of model sharing between Central NWDAF and Local NWDAF should be aligned with in key issue #5.

1.	The NF sends Nnwdaf_AnalyticsInfo_Request to the NWDAF. The request message includes Analytics ID, Target of Analytics Reporting, Analytics Filter Information and Analytics Reporting Information.

2.	The NWDAF 1 determines to perform federated learning for the received Analytics request if it is locally configured to perform federated learning for the requested Analytics ID (e.g. based on the operator policy). The NWDAF 1 also determines to act as Central NWDAF for federation learning for the requested analytics in step 1 if it has Federated Learning Model Aggregation capability (i.e. capability of aggregating local training models and calculating the global model from then).

Editor's note:	How the central NWDAF decides to initiate an FL round i.e. trigger conditions to initiate an FL round is FFS.

Editor's note:	It is FFS how the NWDAF 1 performs subsequent operations for federated learning when it contains only AnLF.

NOTE 1:	The Central NWDAF in this solution is assumed to host both MTLF and AnLF logical functions.

3.	The NWDAF invokes the Nnrf_NFDiscovery_Request service operation from the NRF to find other NWDAFs that are able to perform federated learning as local NWDAFs for the Analytics request in step 1. The Nnrf_NFDiscovery_Request message includes Analytics ID in step 1, ML Model Filter Information according to the Analytics Filter Information received in step 1 and ML model sharing capability that is required to perform Federation Learning.

4.	The NRF returns multiple local NWDAF candidates matching the requested capabilities and Analytics ID.

5.	The central NWDAF requests the local NWDAF candidates to join the federated learning group for the Analytics ID received in step 1. The request message includes Central NWDAF information (e.g. NWDAF ID and address), S-NSSAI, Analytics ID and ML Model Filter Information for the federated learning operation, etc.

6.	The local NWDAF candidates reply to the NWDAF with the indication on whether to accept the federation learning group participation request or not.

7.	Upon receiving the responses from local NWDAF candidates in step 6, the central NWDAF determines a federated learning group consisting of NWDAFs that indicate to accept to the federation learning group participation request in step 6. The central NWDAF creates federated learning group information that includes Analytics ID, list of local NWDAF information (identifiers and addresses of NWDAFs), ML Model Filter Information, ML model information (i.e. the ML model file address), Central NWDAF address as target address where local training model is sent, transmission expiration time information, and local learning processing rule information.

The local learning processing rule information includes either local processing rule ID or local ML model training rule e.g. how many iterations for local model training should be performed for one epoch before providing the local model to the Central NWDAF.

NOTE 2:	The local processing rule ID refers to a pre-agreed operation rule for local ML model training in federated learning operation among NWDAFs.

The transmission expiration time information indicates a deadline before which the local ML model should be transmitted to the Central NWDAF to generate global ML model for an epoch. For example, the deadline can be represented by timestamp and timer value respectively indicating start time of federated learning operation and expiry time period.

8.	The central NWDAF provides the local NWDAFs (i.e. federated learning group member local NWDAFs) with federated learning operation information that includes Analytics ID, ML model information, Central NWDAF address as target address where local training model is sent, transmission expiration time information, and local learning processing rule information.

9.	Local NWDAFs acknowledge to the central NWDAF to indicate if the local NWDAF is able to perform federate learning according to the federated learning operation information.

6.22.3	Impacts on existing nodes and functionality

NWDAF:

-	Support creation of federated learning group with acting as NRF service consumer to discover and select NWDAFs having required capability to perform federated learning.

-	Support to share ML model with other NWDAFs and, especially for Central NWDAF, aggregate ML models provided by other NWDAFs.

NRF:

-	Support new parameters in NF profile provided by NWDAF.

-	Support new discovery parameters related to federated learning group creation.

6.23	Solution #23: Support of federated learning for model training

6.23.1	Description

Model training using Federated learning is useful in scenarios where data may not be easily collected from Data Producer NF. For example, there may be data privacy/data security requirements where data collection from data producer NFs is not feasible or there can be high signalling load if data is collected from Data Producer NFs that have limited resources available (e.g. NFs supporting a local edge network).

Federated learning solves this problem by locally training an ML model using data that are locally available and distributing local model parameters to a central model training function that aggregates the model parameters and generates a central ML model.

The following architecture is proposed to support model training using federated learning.

Figure 6.23.1-1: Architecture supporting model training using federated learning

The main functions of the federated learning architecture is as follows:

-	A central MTLF (that supports FL aggregation) that configures one or more local MTLF(s) (that supports FL participation) with the data that are required to be collected from one or more (local) Data Producer NFs and generates an aggregated ML model based on ML model parameters received from an ML model trained locally from each local MTLF.

-	One or more local MTLF(s) that support ML model training by collecting data from one or more Data Producer NFs available locally.

-	A central Analytics Logical Function (AnLF) that discovers the Central MTLF supporting federated learning for model training since not all MTLFs will support Model Training using federated learning. The AnLF can discover the central MTLF on per Analytic ID/Analytic Filter information basis. The NRF includes information in the NF profile whether an MTLF support FL aggregation or FL participation.

Once the AnLF discovers the Central MTLF that suppors FL aggregation, the AnLF will request an ML model for an Analytic ID including ML model filter information that can include an S-NSSAI, area of interest, DNAI and other information. The Central MTLF based on the Analytic ID requested and the ML model filter information determines that the requested ML model requires federated learning and discovers one or more local MTLFs for each data (i.e Event ID) that cannot be collected directly from a Data Producer NF. The central MTLF discovers the local MTLF(s) from the NRF and configures the local MTLF to train a local ML model indicating the data required to be collected by one or more Data Producer NF(s). The central MTLF then retrieves local ML model parameters from each local MTLF and aggregates gradients/loss factors generating a central ML model that is sent to the AnLF.

6.23.2	Procedures

A detailed procedure is provided below:

Figure 6.23.2-1: Support of model training using federated learning

0.	An AnLF NWDAF receives a request from a consumer NF. The request includes an Analytics ID and analytic filter information as described in TS 23.288 [5].

1.	[conditional]The AnLF determines the central MTLF. The AnLF may determine that a Central MTLF supporting federated learning is required.

2.	[conditional]The AnLF discovers the central MTLF from the NRF by invoking an Nnrf_NFDiscovery_Request including the Analytic ID, service area, MTLF that supports FL aggregation capability.

NOTE:	The service area may be needed in case in a network deployment a specific service area, the MTLF is configured to support model training using federated learning.

3.	[conditional]The AnLF receives from the NRF the NF ID of the central MTLF.

4.	[conditional]The AnLF sends a subscription request to the central MTLF including Analytic ID and ML model filter information as described in TS 23.288 [5].

5.	The central MTLF determines that federated learning is required. The MTLF may determine such information if:

-	Data (corresponding to an Event ID) cannot be collected directly from an NF (based on local configuration)

-	The request for analytics information is for a DNAI where federated learning is required.

-	The request for analytics information is for a service area where federated learning is required.

6.	The central MTLF discovers the local MTLF for federated learning. The determination is based on the training data (i.e. Event ID) required by the local MTLF to train the local ML Model.

7.	The central MTLF discovers the local MTLF from the NRF by invoking an Nnrf_NFDiscovery_Request including the Analytic ID, service area and an indication that the MTLF supporting FL participation for ML model training.

8.	The AnLF receives from the NRF the NF ID of the local MTLF(s).

9a.-9b.	The central MTLF configures the local MTLF(s) to train a local ML model using data from one or more data producer NFs corresponding to the Event ID(s) required. The central MTLF includes in the request to the local MTLF the Analytic ID of the ML model required. The procedure for local configuration is shown in Figure 6.23.2-2.

10a.10b.	Each local MTLF collects data and locally trains an ML model

11a-11b.	The central MTLF and local MTLF(s) exchange ML model parameters that allows aggregation of gradients and loss factors from each local ML model. Steps 11a and 11b are ML model specific and out of scope of 3GPP.

NOTE:	Steps 10a through 11b are iterative and continue until the central MTLF determines that the generated ML model can be used by the AnLF (e.g. reached the required confidence level).

12.	The MTLF updates its central trained model using the aggregated model parameters

13.	The MTLF indicates to the AnLF that a trained model is available.

Figure 6.23.2-2: Support of FL between central and local MTLFs

1.	The central MTLF determines if horizontal or vertical FL takes place based on the analytic ID.The Central MTLF determines th local MTLF(s) for federated learning as described in steps 6-8 of Figure 6.23.2-1. The central MTLF determines if horizontal or vertical FL takes place based on the analytic ID.

2.	The Central MTLF sends a request for federated learning to MTLF 1 where the request includes, an Analytic ID of the model required, one or more of the following Event ID(s) needed, service area.

3.	The MTLF aggregator repeats step 2 for MTLF 2.

5.	MTLF 1 performs data collection and provides model parameters to the Central MTLF aggregator.

6.	MTLF 2 performs data collection and provides model parameters to the Central MTLF.

7.	The Central MTLF calculates aggregated model parameters.

6.23.3	Impacts on services, entities and interfaces

-	Discovery and selection of local MTLF for model training using federated learning

6.24	Solution #24: Horizontal Federated Learning among Multiple NWDAFs

6.24.1	Description

This solution is proposed to address Key Issue #8: Supporting Federated Learning in 5GC.

Current enablers for network automation architecture by NWDAF still faces some major challenges as follows:

-	User data privacy and security (protected by e.g. GDPR) has become a worldwide issue, it is also difficult for NWDAF to collect UE level network data.

-	With the introduction of MTLF in Rel-17, various data from wide area is needed to train an ML model for NWDAF containing MTLF. However, it is difficult for NWDAF containing MTLF to collect all the raw data from distributed data source in different areas, because heavy signalling load may exist if it centralized all the data into the NWDAF containing MTLF.

In order to address the challenges, Federated Learning (also called Federated Machine Learning) technique in NWDAF containing MTLF could be adopted to train an ML model, in which there is no need for raw data transferring (e.g. centralized into one NWDAF) but only need for cooperation among multiple NWDAFs (MTLF) distributed in different areas i.e. sharing of ML model and of the learning results among multiple NWDAFs (MTLF).

During the FL training process, the Server NWDAF can inform the training status to the consumer based on the consumer’s request. The consumer could modify subscription to NWDAF for new model requirement. The Server NWDAF will update or terminate the FL training process accordingly.

This solution is mainly focusing on horizontal Federated Learning among Multiple NWDAFs, whose procedures are illustrated in the following clauses.

6.24.2	Procedures

6.24.2.1	General procedure for Federated Learning among Multiple NWDAF Instances

Figure 6.24.2.1-1: General procedure for Federated Learning among Multiple NWDAF Instances

1-3.	Client NWDAF registers its NF profile (Client NWDAF Type (see clause 5.2.7.2.2 of TS 23.502 [3]), Address of Client NWDAF, Support of Federated Learning capability information, Analytics ID(s)) into NRF.

4-6.	Server NWDAF discovers one or multiple Client NWDAF instances which could be used for Federated Learning via the NRF to get IP addresses of Client NWDAF instances by invoking the Nnrf_NFDiscovery_Request (Analytics ID, Support of Federated Learning capability information) service operation.

It is assumed an Analytics Id is preconfigured for a type of Federated Learning. Thus, the NRF can realize the Server NWDAF is requesting to perform federated learning based on the pre-configuration. And the NRF responds to the central NWDAF the IP address of multiple NWDAF instances which support the Analytics Id.

NOTE:	The analytic ID(s) supporting Federated Learning can be configured by operator.

Based on the response from NRF, Server NWDAF selects which NWDAF clients will participate.

7.	Server NWDAF sends a request to the selected Client NWDAFs that participate in the Federated learning including some parameters (such as initial ML model, data type list, maximum response time window, etc.) to help the local model training for Federated Learning.

NOTE:	This step should be aligned with the outcome of KI#5.

8.	Each Client NWDAF collects its local data by using the current mechanism in clause 6.2 of TS 23.288 [5].

9.	During Federated Learning training procedure, each Client NWDAF further trains the retrieved ML model from the server NWDAF based on its own data, and reports the results of ML model training to the Server NWDAF, e.g. the gradient.

10.	The Server NWDAF aggregates all the local ML model training results retrieved at step 9 such as the gradient to update the global ML model.

11a.	Based on the consumer request, the Server NWDAF updates the training status (an accuracy level) to the consumer using Nnwdaf_MLModelProvision_Notify service periodically (one or multiple rounds of training or every 10 min, etc.) or dynamically when some pre-determined status (e.g., some accuracy level) is achieved.

11b.	[Optional] Consumer decides whether the current model can fulfil the requirement e.g., accuracy and time. The consumer modifies subscription if the current model can fulfil the requirement.

11c.	According to the request from the consumer, the Server NWDAF updates or terminates the current FL training process.

12.	[If the FL procedure continues] The Server NWDAF sends the aggregated ML model information (updated ML model) to each Client NWDAF for next ground model training.

13.	Each Client NWDAF updates its own ML model based on the aggregated model information (updated ML model) distributed by the Server NWDAF at step 12.

NOTE:	The steps 8-13 should be repeated until the training termination condition (e.g. maximum number of iterations, or the result of loss function is lower than a threshold) is reached.

After the training procedure is finished, the globally optimal ML model or ML model parameters could be distributed to the Client NWDAFs for the inference.

6.24.2.2	Procedure for usage of Federated Learning in Abnormal Behaviour

Figure 6.24.2.2-1: Procedure for usage of Federated Learning in Abnormal Behaviour

1.	A Server NWDAF determines to train an Abnormal Classifier using the input data defined in Table 6.7.5.2-1 of TS 23.288 [5]). The Server NWDAF discovers two Client NWDAFs i.e. Client NWDAF 1 in Area A, Client NWDAF 2 in Area B will participate as defined in figure 6.24.2.1-1.

2.	Client NWDAF 1 and Client NWDAF 2 collect raw data from SMF and AMF, respectively. The data collection procedures from AMF and SMF are defined in step 2-3, clause 6.7.5.4 of TS 23.288 [5].

3-4.	Client NWDAF 1 and Client NWDAF 2 collect Exception information from AF, respectively. The Exception information is defined in clause 6.7.5.2 of TS 23.288 [5].

Repeat from step 5-8 until the training termination condition (e.g. maximum number of iterations) is fulfilled:

5.	During Federated Learning training procedure, based on the local raw data, Client NWDAF 1 and Client NWDAF 2 further train the retrieved Abnormal Classifier from the Server NWDAF, respectively.

6.	Client NWDAF 1 and Client NWDAF 2 respectively report the information of local training results for Abnormal Classifier (e.g. volume of the local dataset, parameters for update of the Abnormal Classifier) to the Server NWDAF.

7.	Server NWDAF aggregates all the local training results for Abnormal Classifier to derive the aggregated Abnormal Classifier information.

8.	Server NWDAF sends the aggregated Abnormal Classifier information to Client NWDAF 1 and Client NWDAF 2, and then, Client NWDAF 1 and Client NWDAF 2 update the its own Abnormal Classifier based on the aggregated Abnormal Classifier information distributed by the Server NWDAF.

After the training procedure is finished, the globally optimal Abnormal Classifier or Abnormal Classifier parameters could be distributed to the Client NWDAF 1 and Client NWDAF 2 as well as the new Client NWDAFs, if requested, for the inference.

6.24.3	Impacts on existing nodes and functionality

NWDAF:

-	Registration into NRF;

-	New Data Analytics (i.e. new Analytics ID) for Federated Learning, which can be configured by operator:

-	As a server NWDAF:

-	Discover request to NRF with the analytics id corresponding to a federated learning

-	Client NWDAF instances discovery;

-	Initial Federated Learning parameters determination and distribution;

-	Collection of local ML model information from the Client NWDAF instances;

-	Aggregates local model information to update ML model, e.g. for gradient calculation, reporting to the Server NWDAF.

-	Update the training status to Consumer (AF/NF/OAM) periodically or dynamically.

-	As a Client NWDAF,

-	Receive aggregated model information (updated ML model) from Server NWDAF and perform local ML model update.

NRF:

-	Server NWDAF and Client NWDAF registration and discovery.

6.25	Solution #25: Outdoors Advertisement use case with finer granularity location information

6.25.1	Description

This solution addresses Key issue #9: Enhancement of NWDAF with finer granularity of location information.

In the LCS architecture, the LCS can provide LCS client location information including geographic location, speed, heading, accuracy, timestamp, and so on. Such information precisely represents the location status of a single UE or a group of UE, which can help NWDAF to conduct some valuable statistics and prediction analyses. Therefore, under the permission of users' consent and privacy checking, it is desirable to obtain location information from LCS architecture in line with consumer requirements to help NWDAF make better analyses and predictions.

Outdoors advertisements statistics business is a potential scenario combining NWDAF and LCS architectures. In this scenario, the positioning information of an uncertain group of UE within a certain area needs to be counted. These amount of UEs is changing.

In this scenario, the NWDAF consumers can provide clear statistical requirements including the location of a specific billboard, the period to be calculated, the counting distance range, the accuracy information, and so on. Then NWDAF provides billboard location information, statistical period, number of UE passing by, the proportion of valid viewers, average staying time of viewers, proportion of UE staying longer than the specified time, etc. Thus, a new analytic ID needs to be designed.

Two categories of outdoor advertisements will be studied, expressways advertisements and street advertisements.

For large outdoor advertisements, e.g. roadside advertisements on expressways, advertising information on the billboard can be seen far away. In this way, the granularity of UE location information required by NWDAF may be coarser, such as around 400m near billboards. Therefore, NWDAF only counts the number of users driving towards a correct and fixed direction. Furtherly, the users whose staying time reach a threshold are counted as valid viewers of the advertisement.

For small outdoor advertisements, e.g. street or neighbour advertisements, people can see the contents shown on ads when they are close to the billboard. In this way, the UE location information required by NWDAF will be finer granular, such as around 5m near the billboard. At first, NWDAF conducts preliminary screening on the speed and heading information with ignoring users who move at a fast speed. Then, NWDAF determines the duration of the users and counts the number of people passing by and valid viewers in this certain area.

In order to solve the aforementioned issue, this solution proposes a scheme for providing statistics/prediction service under the measuring outdoors advertisements viewers scenario.

6.25.2	Input Data

The input data for the analytics is described in Table 6.25.2-1.

Table 6.25.2-1: Input data for measuring the number of outdoors advertisements viewers usage

| Information | Source (NOTE) | Description |
| --- | --- | --- |
| Timestamp | LCS architecture | A timestamp is associated with the collected information. |
| Location estimate |  | The geographical location of UE |
| Velocity estimate |  | Including speed and heading information of UE |
| Accuracy |  | Including horizon accuracy and vertical accuracy |
| Age of location |  | The lifetime of location information |

NOTE:	From which network function to acquire the location information will have some coordination with the FS_eLCS_Ph3 study.

6.25.3	Output Analytics

The output analytics of NWDAF is defined in Table 6.25.3-1 and Table 6.25.3-2.

Table 6.25.3-1: Measuring the number of outdoors advertisements viewers usage statistics

| Information | Description |
| --- | --- |
| Core location | The location of the outdoor advertisement billboard |
| Collection period | The time during the beginning and the end of UE location information collection (e.g. 1 hour or 1 day) |
| Total number of users | The total number of users pass by during the collection period |
| Viewers proportion | The proportion of valid viewers among the total number of users |
| Average staying time | The average staying time among all the valid viewers staying time |
| Staying time proportion | The proportion of viewers whose staying time longer than a specific time (e.g. staying timer longer than the 20s) |

Table 6.25.3-2: Measuring the number of outdoors advertisements viewers usage prediction

| Information | Description |
| --- | --- |
| Core location | The location of the outdoor advertisement billboard |
| Predicted Collection period | The time during the beginning and the end of UE location information collection (e.g. 1 hour or 1 day) |
| Predicted Total number of users | The total number of users pass by during the collection period |
| Predicted Viewers proportion | The proportion of valid viewers among the total number of users |
| Predicted Average staying time | The average staying time among all the valid viewers staying time |
| Predicted Staying time proportion | The proportion of viewers whose staying time longer than a specific time (e.g. staying timer longer than the 20s) |

6.25.4	Procedures

Figure 6.25.4-1 depicts a procedure for the Outdoor Advertisement use case analytics provided by NWDAF.

Figure 6.25.4-1: Outdoors Advertisement use case analytics provided by NWDAF

1.	The consumer requests or subscribes to analytics information on the Outdoors Advertisement use case provided by NWDAF. The parameters are carried in the request including the location of a specific billboard, category of advertisement, the period to be counted, the counting distance range, the preferred granularity of the counting distance range, and so on.

-	an Analytics subscription or request (New Analytics ID, Analytics Filter information = AOI, Analytics Reporting information = Analytics target period 1 or prediction target period 2) to the NWDAF by Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request.

The consumer may include multiple sets of parameters in order to provide different combinations of "Location information" and "Analytics target period" when requesting Outdoors Advertisement analytics.

2.	The NWDAF collects the UE location information data from the LCS architecture.

NOTE:	UE location information collection principles will be coordinated with FS_eLCS_Ph3 study.

3.	The NWDAF conducts preliminary screening based on the category of advertisement and then classifies statistics through time stamps and other information. The UE can be counted only when it enters the counting distance range and its location information meets the requirements by the rules of the category of advertisement and NWDAF consumer.

The number of users passing by and the valid viewers can be counted:

-	For the expressway advertisements, users whose geographical location is in a large range of the requested counting distance can be counted as users passing by. The users whose speed and heading meet the requirements are counted as valid viewers of the advertisement. The duration between valid viewers entering and leaving the range requested in NWDAF consumers will be regarded as staying time.

-	For the street advertisements, users whose geographical location is in a smaller range of the requested counting distance can be counted as users passing by. The users who have a limited speed are counted as valid viewers of the advertisement. The duration between valid viewers entering and leaving the range requested in NWDAF consumers will be regarded as staying time.

The NWDAF verifies whether the triggering conditions are met and derives the requested analytics. The NWDAF can detect the need for notification based on comparing the requested analytics of the preferred granularity of counting distance range provided by the consumer over the requested Analytics target period.

4.	The NWDAF provides a response or notification on the Outdoors Advertisement use case to the consumer. The details for UE mobility analytics provided by the NWDAF are defined in the output analytics information.

6.25.5	Impacts on existing nodes and functionality

NWDAF:

-	The NWDAF collects data from LCS architecture and analyses measuring the number of outdoor advertisements viewers.

-	The new Analytic ID supporting the Outdoors Advertisement use case needs to be supported.

Editor's note:	The name of new analytic ID is FFS.

6.26	Solution #26: Finer granularity of location information based on cell sequence

6.26.1	Description

Location information with cell/TA granularity is not enough for some vehicle use cases. For example, while sending warning messages to vehicles running on the road of traffic jam or traffic accident, we need further location information of direction to determine whether the vehicles are running on the up lane or down lane of the same road.

To decide the direction information, we can use the information of cell sequence that the vehicle has went through. Unlike traditional UEs (e.g. mobile phones carried by people) moving irregularly, vehicles can only run forward (cannot run back and forth) along the road so vehicles driving in the same direction of the road tend to go through the same sequence of cells.

6.26.2	Procedures

Firstly, a UE Location order indicator should be added in the Analytics Filter information when consumer NF would like to request or subscribe the UE Mobility analytics. The UE Location order indicator indicates the NWDAF should derives the UE Mobility analytics in a UE Location order.

UE mobility analytics is specified in clause 6.7.2 of TS 23.288 [5]. The following table is the output analytics of UE mobility statistics. Multiple UE locations can be provided in the indicated time slot. However, there is no clear indication about the order of those locations (cells). To determine the direction of location information using cell sequence, we need to add a new NOTE 4 as follows to make the multiple locations are in the right order. Other procedures such as how to subscribe/notify this location information are the same as clause 6.7.2 of TS 23.288 [5].

Table 6.7.2.3-1: UE mobility statistics

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, SUPI (see NOTE 1). |
| Time slot entry (1..max) | List of time slots during the Analytics target period |
| > Time slot start | Time slot start within the Analytics target period |
| > Duration | Duration of the time slot |
| > UE location (1..max) | Observed location statistics (see NOTE 2, 4) |
| >> UE location | TA or cells which the UE stays (see NOTE 3) |
| >> Ratio | Percentage of UEs in the group (in the case of a UE group) |

NOTE 1:	When target of analytics reporting is an individual UE, one UE ID (i.e. SUPI) will be included, the NWDAF will provide the analytics mobility result (i.e. list of (predicted) time slots) to NF service consumer(s) for the UE.

NOTE 2:	If Visited AOI(s) was provided in the analytics request/subscription, the UE location provides information on the observed location(s) that the UE or group of UEs had been residing during the Analytics Target Period.

NOTE 3:	When possible and applicable to the access type, UE location is provided according to the preferred granularity of location information.

NOTE 4:	If target of analytics reporting is an individual UE or multiple UE locations are provided, UE locations will be in the order of which the UE passes through.

To implement the order of the UE locations in the UE Mobility analytics, this solution proposes two alternative solutions:

-	Alt#1, update the UE Mobility analytics i.e. add a list of timestamps for each UE location in the “> UE location (1..max)” during the current “duration” parameter;

-	Alt#2, does not update the UE Mobility analytics but proposes that NWDAF does not aggregate the UE locations in a long duration but provides the UE locations one by one in their own time period, i.e. the “UE location (1..max)” in the UE Mobility analytics has only one UE location which indicates the UE is located in this UE location in the duration from the time slot start.

Editor's note:	It is FFS which alternative will be selected for the normative standards.

6.26.3	Impacts on existing nodes and functionality

Consumer NF (e.g. AMF):

-	Should include a UE Location order indicator into the Analytics Filter when it would like to request/subscribe the UE Mobility Analytics in a UE Location order.

NWDAF:

-	When NWDAF receives a UE location order indicator in the Analytics Filter to request/subscribe the UE Mobility Analytics:

-	Alt#1, the NWDAF adds a list of timestamps or orders for each UE location in the “> UE location (1..max)” during the current “duration” parameter;

-	Alt#2, NWDAF does not aggregate the UE locations in a long duration but provides the UE location one by one in their own time period, i.e. the “UE location (1..max)” in the UE Mobility analytics has only one UE location which indicates the UE is located in this UE location in the duration from the time slot start.

6.27	Solution #27: Relative Proximity Analytics

6.27.1	Description

In Rel-17, NWDAF location analytics are only available with the coarse resolution of Tracking Area (TA) level or cell level, so KI#9 description in cl. 5.9 requests solutions for location analytics with finer granularity. Furthermore, location information is mainly related to the absolute positioning of a UE and relative proximity of a UE versus other UEs is currently lacking in input data acquired by the NWDAF or the output data provisioned by the NWDAF. Relative proximity information can thus be leveraged by NWDAF to provide location information with finer granularity than TA/cell. In this solution, we propose a set of possible procedures for the NWDAF to assist a consumer NF to more accurately localize a cluster (or a set) of UEs via provisioning statistics or prediction information related to their relative proximity. For example, this analytics type may help the consumer improve the location estimation accuracy of a UE by using proximity information from nearby UEs, or it may help the consumer identify UEs in the vicinity of another UE.

Similar to other analytics described in TS 23.288 [5], this solution describes input data (see Tables 6.27.1-1 and 6.27.1-2), output analytics in the form of statistics and predictions (see Tables 6.27.1-3 and 6.27.1-4), and procedures (see cl. 6.27.2) to support relative proximity analytics.

Input Data to the NWDAF from different sources

Table 6.27.1-1: Information collected from OAM

| Information | Source | Description |
| --- | --- | --- |
| Per UE information | OAM |  |
| > Speed |  | UE Speed |
| > Orientation |  | UE Orientation |

Table 6.27.1-2: Information collected via DCAF/ NEF

| Information | Source | Description |
| --- | --- | --- |
| Proximity Attribute | DCAF / NEF | Characterise a set of UEs in relative proximity based on the criteria set |
| > Number of UEs |  | Total number of UEs that fulfil a proximity criteria |
| > Timestamp |  | A time stamp of time that the proximity attribute derived. |
| > Application ID(s) |  | Identifying the applications(s) providing this information |
| > List of UE IDs |  | UE IDs that fulfil a proximity criteria. |
| > Other attribute(s) |  | Other attributes the set of UEs in proximity fulfil as a collective attribute (e.g. destination, route, average speed, time of arrival) |
| > Confidence |  | Confidence on relative proximity data |

Editor's note:	Whether the list of UE IDs that fulfil a proximity criteria is provided by UE to DCAF or determined by DCAF is FFS.

Output analytics from the NWDAF to the consumer NF

Table 6.27.1-3: Relative proximity statistics

| Information | Description |
| --- | --- |
| UE group ID or set of UE IDs | Identifies a group of UEs or a set of UEs, e.g. internal group ID, external group ID, list of SUPIs, list of GPSIs or other external UE IDs. |
| Time slot entry (1..max) | List of time slots during the Analytics target period |
| > Time slot start | Time slot start within the Analytics target period |
| > Duration | Duration of the time slot |
| > UE proximity attribute | Observed proximity data |
| >> relative proximity information | Observed information of proximity changes |
| >> Sampling Ratio | Percentage of UEs accounted based on proximity criteria |

Table 6.27.1-4: Relative proximity predictions

| Information | Description |
| --- | --- |
| UE group ID or set of UE IDs | Identifies a group of UEs or a set of UEs, e.g. internal group ID, external group ID, list of SUPIs, list of GPSIs or other external UE IDs. |
| Time slot entry (1..max) | List of predicted time slots |
| >Time slot start | Time slot start time within the Analytics target period |
| > Duration | Duration of the time slot |
| > UE proximity attribute | Predicted proximity data |
| >> relative proximity information | Predicted information of proximity changes |
| >> Confidence | Confidence of this prediction |
| >> Sampling Ratio | Percentage of UEs accounted based on proximity criteria |

6.27.2	Procedures

Figure 6.27.2-1: Procedure for delivery of relative proximity analytics

1.	The Consumer NF sends a request to the NWDAF for analytics related to relative proximity, using either the Nnwdaf_AnalyticsInfo or Nnwdaf_AnalyticsSubscription service.

The Analytics ID is set to relative proximity information. The target for analytics reporting can be a single UE, group of UEs (e.g. UE1 and UE2 in Figure 6.27.2-1) or any UE. Analytic filters are set to indicate the proximity range or other criteria to be considered for relative proximity. This can be in the form of an area of interest, specific directions of interest, a ranging distance, or other forms of indication. Other analytics filters can also be set to indicate a minimum or maximum number of UEs to be accounted for relative positioning. Other attributes can be also indicated as part of analytics filters, e.g. defining certain velocity, average speed, orientation or mobility trajectory to be accounted for relative positioning. Furthermore, analytics filters can be set to indicate S-NSSAI, DNN, analytics target period or preferred level of accuracy of the analytics.

The Consumer NF can request statistics or predictions or both for a given time window.

2-5.	If the request is authorized, and in order to provide the requested analytics, the NWDAF may subscribe to OAM services to retrieve relevant information to proximity analytics. The NWDAF may collect MDT input data per individual UE from OAM. An example set of information to be provided to the NWDAF is defined in Table 6.27.1-1.

6-7.	For relative proximity information, if the request is authorized, and in order to provide the requested analytics, NWDAF may follow the UE Input Data Collection Procedure via the DCAF. DCAF may collect proximity data directly from the UE Application.

Editor's note:	Whether and how DCAF can collect proximity data is FFS

NOTE 1:	The UE data collection procedure should be based on clause 6.2.8 of TS 23.288 [5].

NOTE 2:	Different Application IDs of the same DCAF or different DCAFs may be selected for different UEs, since each DCAF can only determine the proximity information for the UEs that have PDU session between UE and DCAF.

The NWDAF subscribes to the AF services as above invoking either Nnef_EventExposure_Subscribe for untrusted DCAF or Naf_EventExposure_Subscribe service for trusted DCAF with (Event ID = Relative Proximity, Event Filter information, Target of Event Reporting). The target of event reporting and / or Event Filter information is set according to the target of analytics reporting and / or analytics filters set during the step 1 of the procedure.

Event filters can be defined for relative proximity to indicate to DCAF on how to process the data from individual UEs to determine the set of UEs to be accounted for relative proximity.

In the case of a trusted DCAF, the NWDAF may provide the Area of Interest, proximity range or other criteria to the DCAF on the resolution of TAIs or any other finer resolution recognizable by the 5GC. In the case of an untrusted DCAF, NEF translates the requested criteria provided as event filter by the NWDAF into geographic zone identifier(s) or other geographic range identifier(s) or geographic direction identifier(s) that act as event filter(s) for the DCAF.

The DCAF may processes (e.g. anonymize, aggregate and normalize) the data from individual UEs based on Event Filters indicated by the NWDAF to determine the set of UEs to be accounted for relative proximity before notifying that directly (trusted DCAF) or via NEF (for untrusted DCAF) to the NWDAF. An example set of information to be provided to the NWDAF is defined in Table 6.27.1-2.

8.	The NWDAF derives requested analytics.

9.	The NWDAF provide requested relative proximity information to the consumer NF along with the corresponding Validity Period or any Validity Area, Validity Direction of interest or ranging distance, using either the Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Subscribe response, depending on the service used in step 1. An example set of information to be provided to the consumer NF from the NWDAF is defined in Table 6.27.1-3 and Table 6.27.1-4.

10-12.	If at step 1 the consumer NF has subscribed to receive continuous reporting of relative proximity information, the NWDAF may generate new analytics and, when relevant according to the Analytics target period and Reporting Threshold, provide them along with the corresponding Validity Period (or any Validity Area, Validity Direction of interest or ranging distance) to the consumer NF.

6.27.3	Impacts on existing nodes and functionality

NWDAF:

-	An analytics ID for relative proximity information:

-	A set of input data from other 5GC NFs (e.g. DCAF) or OAM.

-	A set of output data to other 5GC NFs.

-	A set of analytics filters.

AF:

-	A set of event exposures and event filters for proximity information.

DCAF:

-	Support to determine proximity information between different UEs.

OAM:

-	Input data related to proximity information.

6.28	Solution #28: Detect and Improve correctness of NWDAF analytics

6.28.1	Description

The quality of an NWDAF machine-learning model may affect the NF performance or status, according to the usage of an NWDAF analytics and the decisions they make with them. A not reliable (or not well-trained) analytics ID may not help a NF to make a precise or correct decision, which may has impact on aspects of network performance. Hence, it is important to identify when an analytics ID requires improvement. This allows a network the NWDAF inference to keep providing an analytics ID output to one or more NWDAF consumers, which leads to a stable network status. In addition, this enables supporting an NWDAF training to become aware of problems in the configuration and/or generation of an analytics ID, which could lead to an unstable network status.

NWDAF derives statistics and/or predictions, based on requests by analytics consumers. The statistics outputs can be considered more accurate than prediction outputs. Hence, correctness improvement analysis can focus on the prediction outputs of NWDAF analytics.

For the detection of a need for correctness of an analytics ID it is needed to introduce Analytics ID performance information, which could be derived by:

a)	Comparing the predicted outputs of an Analytic ID with the real value (i.e., ground truth) of that analytic ID. For instance, the Mean Absolute Error of the predicted analytics ID could be used as an example Analytics ID performance information, and/or

b)	Measuring the impact of the decision of a NF that uses predicted outputs of an Analytic ID. The impact can be calculated according to the change of relevant KPIs of the NF, after the enforcement of a decision that was made, based on a predicted output of an Analytic ID. For example, the Analytics ID performance information can be defined per Analytics ID from the NF who measures the impact of the decision of a NF that uses predicted outputs of the Analytic ID

Some examples as the following:

-	Analytics ID="Service Experience" used by PCF to adjust the QoS parameters for an application or NSSF/AMF to control the number of UEs or PDU Sessions in a network slice: Increased or Decreased service experience value per UE per Application ID from AF;

-	Analytics ID="UE Mobility" used by AMF to adjust the paging strategy: Increased or Decreased number of paging signalling per UE from AMF;

-	Analytics ID="Network Performance" used by PCF to determine the BDT policy: Increased or Decreased Radio Resource Utilization per RAN from OAM.

NOTE 1:	In addition to the Analytics ID performance information feedback by others NF, the NWDAF itself may also evaluate the Analytics ID performance based on the continuous input data from the data sources e.g. the continuous service data as defined in Table 6.4.2-1 for the Analytics ID = "Service Experience".

The above-mentioned Analytics ID performance information is to indicate the execution result of a decided action based on the effect of the analytics ID. It can be calculated at NWDAF by using the analytics IDs statistics outputs. For instance, the predictions of an Analytics ID will be become the statistics of the respective analytics ID in a future point of time. If the relevant KPIs of the NF are in the statistics outputs of another analytics ID which reflects the effects of the action taken by the NF based on the prediction, the NF making decisions may provide the NWDAF the relationship between the above-mentioned Analytics ID performance information and the statistics outputs of the analytics ID (i.e. another analytics ID mentioned above). In this case, the NWDAF calculates the Analytics ID performance information according to the statistics outputs of the analytics ID specified by the NF making decisions based on the prediction.

Alternatively or in conjunction with the NWDAF, the above-mentioned Analytics ID performance information can also be provided by a NF that makes decisions and/or applies decided actions via a feedback, based on the prediction of the analytics ID. The feedback can indicate the execution result of a decided action based on the effect of the predicted analytics ID. For instance, Analytics ID which reflects the effects of the action taken by the NF based on the prediction, the NF making decisions may compare the result with the decision made based on the statistics outputs of the analytics ID and indicates "good/hit" or "not good/hit" to the NWDAF.

The monitoring of network analytics correctness needs to identify the sources of data collection for the KPIs and/or metrics related to the Analytics ID performance information. And then to start the data collection from these data sources. The monitoring functionality can provide indications about the effect of each Analytics ID:

-	Analytics ID Grade Information: information resulting from the calculation of a grade related to the effect of an analytics on the changes in network status after the consumption of analytics.

-	Unstable Analytics ID Information: information resulting from the calculation of a grade related to the effect of an analytics on the changes in network status after the consumption of analytics and identifying that this grade as crossing, deviating from thresholds (e.g., thresholds configured by operator) that denote an analytics has been identified as leading to unstable network status.

The NWDAF with a tracing function can associate information of the quality of usage of the analytics ID, and the configuration at inference and/or training over time for an analytics ID. The NWADF can keep information about last known stable network states, which is a network state at a given point in time when using a given analytics with the most appropriated configuration.

The activation of the tracing and the monitoring of an analytics ID can happen:

-	with the subscription request of the analytics ID by the analytics consumer that is provided to the NWDAF. The request can include information to indicate that the tracing of an analytics ID needs to be started, optionally together with information about the criteria of the correctness of the analytics ID by the analytics consumer. The criteria of correctness includes the acceptable threshold of Analytics ID performance information and the corresponding correctness evaluation. Based on the criteria, the NWDAF can calculate the Analytics ID Grade Information of the corresponding analytics ID.

-	or by the NWDAF itself.

According to the outputs of tracing and monitoring of an analytics ID, e.g., if the analytics ID is unstable (i.e., low performance), the NWDAF needs to refresh the output of the analytic ID and can decide and provide notifications for actions at the Inference and/or the Training functions. The quality of the NWDAF analytics is affected by various factors e.g., the quality and amount of the collected data to train, the configuration of machine learning model etc. Example actions at the Inference Function (AnLF) include:

-	Changing an inference configuration of an analytics ID

-	Selecting a new analytics model of an analytics ID

-	Deactivate an analytics model of an analytics ID

Example actions at the Training Function (MTLF) include:

-	Changing a training configuration of an analytics ID

-	Perform retraining or reselection of an analytics model of an analytics ID

-	Deactivate an analytics model of an analytics ID

The NWDAF can use stored information during tracing of an analytics ID (e.g., on successful configurations of MTLF or AnLF) that have led to a stable or good network status for an analytics consumer, in case that an improvement decision of an analytics ID has been initiated. This information can help the NWDAF in its improvement decision and select improvement action.

The Inference Engine (AnLF) can provide Notifications to the analytics consumers, when it is identified that an analytics ID is unstable or needs improvement or is stable again:

-	a notification for the analytics ID that is unstable (i.e., low performance)

-	a status notification that an analytics ID that is stable (i.e., high performance) and the refreshed output of the Analytic ID.

-	a cooling duration indicates NWDAF needs more time to refresh the output of the Analytic ID. The consumer NF can re-request the same Analytic ID after the cooling duration.

This notification can help the NFs to decide whether they can continue consuming the specific analytics ID (that requires improvement).

6.28.2	Procedures

Monitor correctness

Analytics Consumer (NF)

AnLF

MTLF

Data Sources

4b. NF Feedback for Analytics ID Performance

7. Notification on Analytics ID Status (Unstable/Stable/…)

Improve Correctness

2. Analytics Generation and Notification

1. Analytics Subscription with indication for Analytics Tracing and Monitoring activation

4a. Analytics ID Performance Monitoring

Decision Enforcement (NF)

3. Apply actions decided using analytics

5. Tracing of Analytics ID and Detect Analytics ID Correctness requirement

6. Analytics ID Improvement Actions selection and execution

Figure 6.28.2-1: Procedure to monitor and improve the correctness of NWDAF analytics

1.	The analytics consumer subscribes to the NWDAF service by invoking the Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request service operation and provides an indication that the tracing of an analytics ID needs to be started, optionally together with information about the criteria of the correctness of the analytics ID consumed by the analytics consumer. The NWDAF may indicate the analytics consumer or Decision Enforcement NF (via the analytics consumer) to provide feedback to the NWDAF (AnLF) about the performance information of the corresponding analytics ID in the response message.

2.	The NWDAF (AnLF), generates the analytics output, according to the requested analytics ID and provides the notifications to analytics consumer, by invoking Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsSubscription_Notify service operation (as presented in section 6.1, TS 23.288 [5])

3.	The analytics consumer (NF) may use the received analytics outputs to make decisions that can be applied (or executed) either directly by the same NF (i.e., the analytics consumer) or the analytics consumer (NF) may use the received analytics outputs to make decisions that can be applied by another NF (i.e., Decision Enforcement NF) e.g. when the analytics consumer is PCF.

4.	In the case that the monitoring of the correctness of an analytics ID has been initiated then performance information of the corresponding analytics ID are calculated within the NWDAF and/or with the support of involved NFs.

4a.	The NWDAF can calculate performance information of the analytics ID by deriving statistics of same (i.e. predicted) analytics ID and/or statistics of (one or more other) analytics ID that reflect the effects of the action taken by the NF based on the predicted outputs according to the criteria of the correctness of the analytics ID. The NWDAF collects data from the analytics consumer (NF) and/or the Decision Enforcement NF to generate the output of the tracing Analytics ID.

4b.	The analytics consumer (NF) can provide the feedback to the NWDAF (AnLF) about the performance information of the corresponding analytics ID, optionally by receiving information by a Decision Enforcement NF (if involved). The analytics consumer NF or the Decision Enforcement NF, can provide feedback of the execution result (e.g. based on changes of relevant KPIs of the NF) of a decided action based on the effect of the predicted analytics ID.

Editor's note:	The feedback details are FFS.

5.	The NWDAF (AnLF) detects whether there is need for improvement of an Analytics ID, which is leading to unstable network status. By comparing monitored analytics ID performance information with the criteria of the correctness of an analytics ID (i.e., that have been provided by the analytics consumer or set in the NWDAF), the NWDAF calculate the Analytics ID grade information. The NWDAF will decide whether the correction is needed according to the Analytics ID grade information. The unstable Analytics ID information already known are also taken into account.

The NWDAF (AnLF) can also associate information about the quality of usage of the analytics ID, and the configuration at inference and/or training over time for an analytics ID, via a tracing function included in the NWDAF (AnLF). The NWDAF (AnLF) can keep information about last known stable network states, which is a network state at a given point in time when using a given analytics with the most appropriated configuration. The above can facilitate the decision making of the improvement process.

Editor's note:	It is FFS what network information or status NWDAF is tracing and how?

6.	In the case that the need for correctness of analytics ID has been detected then an improvement process should be initiated. The decision for improvement of the unstable analytics ID at the NWDAF can be made using analytics ID performance information. The specific improvement action that will be triggered is an internal decision of the NWDAF and can involve the AnLF and/or the MTLF.

-	Example actions at the Inference Function (AnLF) could be: change an inference configuration of an analytics ID, selecting a new analytics model of an analytics ID, Deactivate an analytics model of an analytics ID.

-	Example actions at the Training Function (MTLF) could be: changing a training configuration of an analytics ID, perform retraining or reselection of an analytics model of an analytics ID, deactivate an analytics model of an analytics ID.

The NWDAF can use stored information during tracing of an analytics ID (e.g., on successful configurations of MTLF or AnLF) that have led to a stable or good network status for an analytics consumer to support an improvement decision for an analytics ID.

7.	A notification to the consumer of the NWDAF analytics ID is provided, when it is identified that an analytics ID is unstable or needs improvement. The notification message can include:

-	a notification for the analytics ID that is unstable (i.e., low performance)

-	a status notification that an analytics ID that is stable (i.e., high performance) and the refreshed output of the Analytic ID

-	a cooling duration indicates NWDAF needs more time to refresh the output of the Analytic ID. The consumer NF can re-request the same Analytic ID after the cooling duration.

6.28.3	Impacts on Existing Nodes and Functionality

NWDAF:

-	Monitor the correctness of an analytics ID, calculating performance information of an analytics ID

-	Tracing correctness of an analytics ID and detect whether there is need for improvement of an Analytics ID

-	Decide and apply appropriate action for the improvement of an Analytics ID

-	Provide notification to the consumer of an analytics ID, when it is identified that an analytics ID is unstable or needs improvement.

Consumer NFs:

-	Indicate that the tracing of an analytics ID needs to be started, providing (optionally) information about the criteria of the correctness of the analytics ID

-	Provide feedback to the NWDAF about the performance information of an analytics ID on the execution result (e.g. based on changes of relevant KPIs of the NF) of a decided action based on the effect of the predicted analytics ID.

Decision Enforcement NFs:

-	Provide feedback to the NWDAF or to the Consumer NF about the performance information of an analytics ID and the execution result (e.g. based on changes of relevant KPIs of the NF) of a decided action based on the effect of the predicted analytics ID.

6.29	Solution #29: Detection of ML Model degradation and actions

6.29.1	Description

This is a solution for KI#1, on how to improve correctness of NWDAF analytics and addresses the following item in this KI:

-	How and which information is needed to enhance the ML model provisioning to improve the correctness of NWDAF Analytics.

-	Study mechanisms to detect that degradation on an ML model has happened and whether and which actions should be triggered.

In current ML model provisioning procedure defined in TS 23.288 [5], an NWDAF containing MTLF provides a trained ML model to an NWDAF containing AnLF which invokes the Nnwdaf_MLModelProvision_Subscribe towards the NWDAF containing MTLF.

However, the performance of the trained ML model when used for analytics, e.g. accuracy level or confidence of the predictions using the trained ML model, may not meet the analytics requirement(s) of the NWDAF containing AnLF according to the analytics request(s)/subscription(s) from analytics consumer NF(s). Also the performance of the trained ML model may degrade, due to e.g. changes of network status and changes of UE communication patterns, etc.

This solution considers two aspects of the ML Model degradation problem:

-	Detection of the degradation of an ML model.

-	Triggering of a proper action in case any degradation in an ML model is observed.

The trained ML model needs to be updated by the NWDAF containing MTLF in order to guarantee or improve the correctness of analytics by the NWDAF containing AnLF. Hence, a solution of enhancing existing ML model provisioning procedure is proposed, so that the NWDAF containing MTLF can detect the degradation of an ML model, update the ML model towards the NWDAF containing AnLF accordingly and thus improve the correctness of NWDAF analytics.

6.29.2	Procedures

6.29.2.1	General

The proposed solution consists of two main parts:

-	How to detect whether an ML model has been degraded.

-	Trigger proper action in case ML model degradation has been observed.

Each part is discussed in the following subsections.

6.29.2.2	Detection of ML Model Degradation

Figure 6.29.2.2-1 illustrates an NWDAF containing MTLF subscribing to the monitoring of an ML model at an NWDAF containing AnLF. If any degradation in the ML model is detected, the NWDAF containing AnLF generates a notification to the NWDAF containing MTLF.

In this solution, the NWDAF containing AnLF may start the monitoring process when it starts making use of an ML model, and it registers with the MTLF that is responsible for training/updating this ML model. The NWDAF containing MTLF, due to the registration of monitoring from NWDAF containing AnLF or due to local policies, subscribes to the NWDAF containing AnLF for receiving notifications of the detection of degradation of the ML Model.

When the subscription request is received, the NWDAF containing AnLF is assumed to be in possession of possible ML model’s associated information, included in the ML model file, that can be used to support the AnLF performing the monitoring of the ML model. The NWDAF containing AnLF may start the monitoring process when it starts making use of the ML model. NWDAF containing AnLF monitors the performance of the ML model and notifies NWDAF containing MTLF if any degradation is detected.

Figure 6.29.2.2-1: Monitoring an ML model and report degradation if detected.

6.29.2.3	Act Upon ML Model Degradation

In case of detecting degradation of an ML model which has been monitored, an NWDAF containing AnLF notifies the subscribed NWDAF containing MTLF of this detection. The NWDAF containing MTLF may not declare an ML model as "degraded" until enough degradation detection notifications have been received. When the NWDAF containing MTLF determines that an ML model is "degraded", it marks it with an indication in the ML model information (see 3GPP TS 23.288 [5] clause 6.2A.2 for existing contents).

It is then up to the NWDAF containing MTLF to determine the actual actions to be taken. These actions include, e.g.:

-	Trigger a re-training process with an extended data set, including that the NWDAF containing MTLF may need to collect new data (e.g., statistics) from, e.g., ADRF.

-	In case of existing more than one ML model for an Analytics ID, based on the records of ML model performance metrics, another ML model might be chosen without re-training any of the existing ML models.

6.29.2.4	Procedure for the registration of ML model monitoring

Figure 6.29.2.4-1 illustrates the procedure by which an NWDAF containing AnLF registers with an NWDAF containing MTLF that it is starting to monitor the degradation of an ML model. A new Nnwdaf_MLModelMonitor_Register service operation is used for that purpose.

Figure 6.29.2.4-1: Procedure for ML model monitoring registration

1.	An NWDAF containing AnLF starts monitoring an ML model, due to start using the ML model for producing analytics

2-3.	The NWDAF containing AnLF sends an Nnwdaf_MLModelMonitor_Register request to an NWDAF containing MTLF (NWDAF containing AnLF NF ID, unique identifier of the ML model, optionally: subscription endpoint of the ML Monitoring service operation). The NWDAF containing MTFL is now aware of the NF ID of the NWDAF containing AnLF that are monitoring that ML model.

When the NWDAF containing AnLF does no longer monitor the ML model, it sends an Nnwdaf_MLModelMontior_Unregister service operation. The NWDAF containing MTLF deletes the associated data.

Editor's note:	It is FFS whether and how AnLF indicates its support of ML model monitoring during the ML model provisioning procedure

6.29.2.5	Procedure for the detection of ML model degradation

Figure 6.29.2.5-1 illustrates the procedure for the detection of ML model degradation. A new Nnwdaf_MLModelMonitor_Subscribe and Nnwdaf_MLModelMonitor_Notify service operations are used for that purpose. An NWDAF service consumer, i.e. an NWDAF containing MTLF, subscribes at another NWDAF, i.e. an NWDAF containing AnLF, to be notified when the performance of the previously provisioned trained MT model degrades.

Figure 6.29.2.5-1: Procedure for ML model degradation detection

0.	An NWDAF containing MTLF receives a trigger for subscribing to the ML model monitor at an NWDAF containing an AnLF. This trigger could be the reception of an Nnwdaf_MLModelMonitor_Register request, an administrative action, or determined by local policy. The NWDAF containing MTLF reads the information registered by the AnLF, such as the NF ID, unique identifier of the ML model, MLModelMonitor subscription endpoint, etc.

1.	The NWDAF containing MTLF sends an Nnwdaf_MLModelMonitor_Subscribe request (Analytics ID, unique identifier of the ML model to be monitored, optionally Reporting Threshold(s)) to an NWDAF containing AnLF subscription endpoint.

2.	The NWDAF containing AnLF sends a response to the NWDAF containing MTLF.

3.	NWDAF containing AnLF starts an internal process for monitoring the degradation of the ML model.

NOTE:	NWDAF containing AnLF can monitor the model performance or accuracy in many ways: e.g. get ground truth value of the predictions, subscribe to NF events that related to the analytics ID, or following the instructions embedded in the model from the MTLF, etc.

4,5	Upon detection of the degradation of the ML model, and if deviation (i.e. deviation of the output analytics using the trained ML model from the actual network running data) is greater than the Reporting Threshold(s) if received in the Subscribe, the NWDAF containing AnLF sends an Nnwdaf_MLModelMonitor_Notify request to the notification endpoint (e.g., the NWDAF containing MTLF). The Notify request should include a Deviation value.

6.	The NWDAF containing MTLF sends a response indicating that it has received ML model monitor notification from respective instance of NWDAF containing AnLF.

7.	The NWDAF containing MTLF adds an indication to the ML Model Information as "degraded".

8.	The NWDAF containing MTLF takes an appropriate action. This can be, e.g., re-training the ML model with additional data.

6.29.2.6	Procedures for ML Model Provisioning

Model provisioning is affected by the proposed solution in a way that provisioning can be postponed if an action might be required, in case of detection of an ML model degradation.

When NWDAF containing MTLF receives a subscription request from a NWDAF’s service consumer for ML model provision, upon accepting the subscription, the NWDAF containing MTLF generates a regular notification request including the required ML model information including an indication of the "degraded" status.

Figure 6.29.2.6-1 shows an enhanced procedure for signalling whether a model is degraded.

Figure 6.29.2.6-1: Procedure for enhanced ML model provisioning

1.	An ML Model Provisioning service consumer (i.e., an NWDAF containing AnLF) sends an Nnwdaf_MLModelProvisioning_Subscribe request for subscribing to a (set of) ML models associated to a (set of) Analytics ID.

2.	The NWDAF containing MTLF sends a response.

3,4.	Prior to sending a notification, the NWDAF containing MTLF determines whether the model is marked as "degraded". Then the NWDAF containing MTLF sends an Nnwdaf_MLModelProvisioning_Notify request (ML model Information, including the file address of the trained ML model and its "degradation" indication status).

5,6.	The NWDAF containing AnLF sends a response and determines, based on the "degraded" indication in the ML Model information, whether it still wants to download the ML model or not.

7.	Later, the NWDAF containing MTLF finishes the re-training and a new ML model becomes available. The MTLF clears the "degraded" indication of the ML Model Information.

8.	The NWDAF containing MTLF sends an Nnwdaf_MLModelProvisioning_Notify request (ML Model Information, addresses of the file address of the trained ML model).

9.	The service consumer acknowledges NWDAF containing MTLF by sending back Nnwdaf_MLModelProvision_Notify response.

The ML Model Information (including the new "degraded" indication) is also available through the Nnwdaf_MLModelInfo_Request service operation.

6.29.3	Impacts on Existing Nodes and Functionality

NWDAF containing AnLF:

-	New functionality for monitoring the degradation of an ML model and registering/unregistering the monitoring of an ML model using the Nnwdaf_MLModelMonitor_Register service operation.

-	New functionality for receiving subscription requests for monitoring the degradation of an ML model (Nwdaf_MLModelMonitor_Subscribe/Unsubscribe) and generating their corresponding notifications (Nnwdaf_MLModelMonitor_Notify).

-	Upon requesting the provisioning of an ML model, new functionality for receiving a notification with the ML model information containing the "degraded" indication in Nnwdaf_MLModelProvisioning_Notify request and response to Nnwdaf_MLModelInfo_Request.

NWDAF containing MTLF:

-	New functionality for registering/unregistering and storing NF ID of NFs which are monitoring of an ML model by receiving the Nnwdaf_MLModelMonitor_Register service operation.

-	New functionality for subscribing to notifications of the degradation of an ML model (Nnwdaf_MLModelMontior_Subscribe/Unsubscribe and Nnwdaf_MLModelMonitor_Notify).

-	New functionality for triggering an NWDAF containing MTLF action upon receiving information of the degradation of an ML model. Such actions may include, starting a procedure for re-training the degraded ML model.

-	New functionality for including a "degraded" indication in the ML model information, Nnwdaf_MLModelProvisioning_Notify request and response to Nnwdaf_MLModelInfo_Request.

  - 6.30	Solution #30: Improve correctness of NWDAF analytics

6.30.1	Description

This is a solution for Key Issue #1: "How to improve correctness of NWDAF analytics".

After MTLF providing one or multiple ML models related to an analytics ID to the AnLF, based on some specific monitoring mechanism, the analytics correctness for the one model or multiple models can be evaluated by the AnLF or MTLF. The analytics correctness evaluation result (e.g. Accuracy in Use or MAE in Use, etc.) will be obtained or reported to the MTLF meanwhile with the model usage scope for each model, which indicates the target scope (area, DNN, S-NSSSAI and target UE(s)) the model is used in the real network.

By updating the model repeatedly, or choosing from the multiple models, the MTLF will get a model with good enough performance e.g. according to analytics correctness evaluation result (e.g. Accuracy in Use or MAE in Use, etc.). The MTLF stores the model information with and its corresponding model usage scope.

When the same AnLF (e.g. due to abnormal loss of model information) or some other AnLF subscribes a trained ML model to the MTLF, if the requested analytics ID and model filter information matches with the model information, then the MTLF provides it to the requested AnLF.

6.30.2	Procedures

6.30.2.1	Procedures for improving model performance based on analytics correctness evaluation of one candidate model

Figure 6.30.2-1: procedures for improving model performance based on analytics correctness evaluation of one candidate model

1.	The NWDAF containing AnLF1 subscribes to a trained ML model associated with an Analytics ID and model filter information by invoking Nnwdaf_MLModelProvision_Subscribe or Nnwdaf_MLModelProvision_Request service operation as defined in TS 23.288[5].

The NWDAF containing MTLF provides a matched ML model information by invoking Nnwdaf_MLModelProvision_Notify or Nnwdaf_MLModelProvision_Response service operation.

2.	The NWDAF containing AnLF1 retrieves analytics based on the obtained ML model and provides the analytics result to the analytics NF consumer, which is described in TS 23.288[5].

During the analytics inference procedure and analytics usage procedure in the real network, the analytics correctness can be evaluated by the AnLF1 or MTLF, e.g. based on monitoring mechanism defined in the solution 1, 3, 4, 5, 6 or others. The analytics correctness evaluation result (e.g. Accuracy in Use or MAE in Use, etc.) will be obtained or reported to the MTLF as well with the model usage scope, which indicates the target scope (area, DNN, S-NSSSAI and target UE(s)) the model is used in the real network.

NOTE:	how does the AnLF and MTLF evaluate analytics correctness should depend on the conclusion of KI#1.

3.	If the analytics correctness is low and doesn't achieves the configured goal, then the MTLF will reselect a new ML model or retrain the provided ML model for the corresponding model usage scope.

4.	The NWDAF containing MTLF provides a new model or the updated model to the NWDAF containing AnLF1, with a corresponding model ID, which uniquely identifies a model instance.

5.	Similar as the step 2, the analytics correctness for the new model or the updated model can be re-evaluated. And then steps 3-5 will be repeated until the analytics correctness is well enough as described in step 6.

6.	If the analytics correctness is good enough e.g. based on configuration in MTLF, then the NWDAF containing MTLF stores the model information and its corresponding model usage scope.

7.	The NWDAF containing AnLF1(e.g. due to abnormal loss of model information) or NWDAF containing AnLF2 subscribes to a trained ML model associated with an Analytics ID and model filter information, which is similar as step 1.

8.	If NWDAF containing MTLF determines the requested analytics ID and model filter information matches with model filter information and the model usage scope, then it will perform step 9 to provide the model information to the AnLF.

9.	The NWDAF containing MTLF provides the matched ML model information determined in step 8 by invoking Nnwdaf_MLModelProvision_Notify or Nnwdaf_MLModelProvision_Response service operation.

6.30.2.2	Procedures for improving model performance based on analytics correctness evaluation of multiple candidate models

Figure 6.30.2-2: procedures for improving model performance based on analytics correctness evaluation of multiple candidate models

1.

The NWDAF containing AnLF1 subscribes to a trained ML model associated with an Analytics ID and model filter information by invoking Nnwdaf_MLModelProvision_Subscribe or Nnwdaf_MLModelProvision_Request service operation as defined in TS 23.288[5].

If there are multiple candidate models matching with the model request in the NWDAF containing MTLF, and optionally NWDAF containing AnLF indicates its support for multiple ML models, then NWDAF containing MTLF provides multiple matched ML models IDs and corresponding model information by invoking Nnwdaf_MLModelProvision_Notify or Nnwdaf_MLModelProvision_Response service operation.

2.	For the multiple obtained ML models, the analytics correctness for each model can be evaluated by the AnLF1 or MTLF, e.g. based on monitoring mechanism defined in the solution 1, 3, 4, 5, 6 or others. The analytics correctness evaluation result for each model (e.g. Accuracy in Use or MAE in Use, etc.) will be obtained or reported to the MTLF as well with the model usage scope, which indicates the target scope (area, DNN, S-NSSSAI and target UE(s)) the model is used in the real network.

3.	Based on the received analytics correctness evaluation result for the multiple candidate ML models, the NWDAF containing MTLF chooses the one(s) whose analytics correctness is good enough e.g. based on configuration in MTLF, and stores the model information and the corresponding model usage scope.

4.	The NWDAF containing AnLF1(e.g. due to abnormal loss of model information) or NWDAF containing AnLF2 subscribes to a trained ML model associated with an Analytics ID and model filter information, which is similar as step 1.

5.	If NWDAF containing MTLF determines the requested analytics ID and model filter information matches with model filter information and the model usage scope, then it will perform step 6 to provide the model information to the AnLF.

6.	The NWDAF containing MTLF provides the matched ML model information determined in step 8 by invoking Nnwdaf_MLModelProvision_Notify or Nnwdaf_MLModelProvision_Response service operation.

6.30.3	Impacts on existing nodes and functionality

NWDAF containing MTLF：

-	Obtains by itself or from the AnLF the analytics correctness evaluation result for each model (e.g. Accuracy in Use or MAE in Use, etc.) and the corresponding model usage scope;

-	Stores the model information with the good enough performance and its corresponding model usage scope.;

-	Provides the model information with good enough performance to the AnLF, based on the analytics ID, model filter information and model usage scope.

6.31	Solution #31: Multiple Analytics outputs based NF action decision

6.31.1	Description

This is a solution for Key Issue #1: "How to improve correctness of NWDAF analytics".

When the consumer NF requests analytics for the prediction and willing to decide the action based on the prediction, there exists a certain risk on incorrect prediction by NWDAF. Specifically, the NWDAF provides a single prediction output as specified in current TS 23.288 [5], the consumer NF only can determine the action based on the single prediction, even if the single prediction has low probability to true (e.g., 0.51) but high probability to fail (e.g., 0.49).

In order to minimize the effect of incorrect analytics on NF action, this solution proposes that NWDAF provides multiple prediction outputs with the certain probability to the consumer NF, and then, the consumer NF takes an action which considers all the possible ground truth from multiple analytics outputs.

6.31.2	Procedures

Figure 6.31.2-1 depicts the procedures for providing multiple analytics to consumer NF.

Figure 6.31.2-1: Procedures for providing multiple Analytics outputs

0.	The NWDAF containing AnLF registers the capability to supports multiple analytics output to NRF.

1a-1b.	The analytics consumer NF discovers the NWDAF containing AnLF via NRF by invoking Nnrf_NFDiscovery Request with the indication of multiple analytics outputs, and selects the serving NWDAF from the candidate NWDAFs.

2a-2b.	The analytic consumer NF requests analytics to the NWDAF by invoking Nnwdaf_AnalyticsInfo_reqeust/Nnwdaf_AnalyticsSubscription_Subscribe service operation with the indication of multiple analytics outputs which includes either "the number of required analytics outputs" or "the lowest probability of analytics", and then, the NWDAF provides the multiple outputs (i.e., prediction results) with the probability (i.e., confidence in output parameter) as indicated by "the number of required analytics outputs" or the outputs which has larger probability than "the lowest probability of analytics" for an Analytics to the analytics consumer NF.

NOTE:	The sum of the probability (i.e., confidence in output parameter) for all the outputs should be less than or equal to 100.

Editor's note:	It is FFS the maximum number of required analytics outputs can be achieved by Maximum number of objects in Analytics Reporting Information.

3.	The analytics consumer NF decides an action which considers all the possible ground truth from multiple analytics outputs.

6.31.3	Impacts on existing nodes and functionality

NWDAF:

-	NWDAF registers the capability to provide multiple analytics output to NRF.

-	In case prediction, NWDAF provides multiple analytics outputs with each probability in confidence field.

NF:

-	Analytics consumer NF decides an action based on multiple NWDAF analytics outputs.

6.32	Solution #32: Enhanced ML model provisioning

6.32.1	Description

This solution addresses key issue #1 "How to improve correctness of NWDAF analytics".

In current ML model provisioning procedure defined in TS 23.288 [5], an NWDAF containing MTLF provides a trained ML model to an NWDAF containing AnLF which invokes the Nnwdaf_MLModelProvision_Subscribe towards the NWDAF containing MTLF.

However, the performance of the trained ML model when used for analytics, e.g. accuracy level or confidence of the predictions using the trained ML model, may not meet the analytics requirement(s) of the NWDAF containing AnLF according to the analytics request(s)/subscription(s) from analytics consumer NF(s). Also the performance of the trained ML model may degrade, due to e.g. change of network status / collected network data.

In those cases, the trained ML model needs to be updated by the NWDAF containing MTLF in order to guarantee or improve the correctness of analytics by the NWDAF containing AnLF.

Hence, a solution of enhancing existing ML model provisioning procedure is proposed, so that the NWDAF containing MTLF can detect the performance requirements or degradation of an ML model, update the ML model towards the NWDAF containing AnLF accordingly and thus improve the correctness of NWDAF analytics.

6.32.2	Procedures

The procedure in Figure 6.32.2-1 is used by an NWDAF service consumer, i.e. an NWDAF containing MTLF, to subscribe at another NWDAF, i.e. an NWDAF containing AnLF, to be notified when the performance of the previously provisioned trained MT model does not meet the analytics requirements or degrade.

NOTE 1:	The service names in the procedure are descriptive and may be updated where applicable during the normative phase.

Figure 6.32.2-1: Procedure of enhanced ML model provisioning

1.	NWDAF1 containing MTLF provisions the same ML model to NWDAF2 containing AnLF, NWDAF3 containing AnLF and NWDAF4 containing AnLF based on the Nnwdaf_MLModelProvision_Subscribe service operations from those NWDAFs, using the procedure described in clause 6.2A.1 of TS 23.288 [5]. The NWDAF2 containing AnLF, as well as NWDAF3 containing AnLF, indicated its capability of providing ML model performance feedback information in the Nnwdaf_MLModelProvision_Subscribe service operation.

2a-2b.	NWDAF1 containing MTLF decides to subscribe to notifications on the trained ML model performance. Based on the indicated capability of providing ML model performance feedback information in the Nnwdaf_MLModelProvision_Subscribe service operation, NWDAF1 containing MTLF invokes the Nnwdaf_MLModelPerformance_Subscribe service operation towards NWDAF2 containing AnLF and NWDAF3 containing AnLF respectively. The Nnwdaf_MLModelPerformance_Subscribe contains the Analytics ID(s) associated with the trained ML Model(s) and optionally Reporting Threshold(s) for trained ML model performance. Here the Reporting Threshold(s) refers to the average and/or variance of the deviation of the predictions (i.e. predicted data) using the trained ML model from the actual network data, in terms of e.g. Mean Absolute Error or Accuracy.

3a-3b.	NWDAF2 containing AnLF (/NWDAF3 containing AnLF) determines whether/when to send notifications on the performance of trained ML model to the NWDAF containing MTLF. When NWDAF2 containing AnLF (/NWDAF3 containing AnLF) detects that the analytics deviation (i.e. deviation of the predictions using the trained ML model from the actual network data) is greater than the Reporting Threshold(s) if received in the Nnwdaf_MLModelPerformance_Subscribe in Step 2, or greater than the Reporting Threshold(s) which is set according to local policy, NWDAF2 containing AnLF (/NWDAF3 containing AnLF) invokes an Nnwdaf_MLModelPerformance_Notify service operation towards NWDAF1 containing MTLF. The Nnwdaf_MLModelPerformance_Notify indicates the analytics deviation which is greater than the Reporting Threshold(s), and optionally the data set for the detected deviation when the deviation occurs. The ML model accuracy (i.e. level of accuracy of the analytics using the trained ML model) may also be included in the Nnwdaf_MLModelPerformance_Notify. The information contained in the Nnwdaf_MLModelPerformance_Notify is shown in Table 6.32.2-1.

Table 6.32.2-1: ML model performance data from the NWDAF containing AnLF

| Information | Source | Description |
| --- | --- | --- |
| Analytics ID | NWDAF | The Analytics ID(s) associated with the trained ML Model(s). |
| Analytics deviation | NWDAF | Deviation of the predictions using the trained ML model from the actual network data, in terms of e.g. Mean Absolute Error or Accuracy. |
| ML model accuracy | NWDAF | Level of accuracy of the analytics using the trained ML model. |
| Data set for the detected deviation | NWDAF | The input data, output predictions and actual network data when the deviation occurs. |
| Timestamp | NWDAF | A time stamp associated to the ML model performance information provided by the NWDAF. |
| NWDAF Instance | NWDAF | The ID of the NWDAF which provides the ML model performance information. |

NOTE 2:	The address (e.g. URL) of the data set for the detected deviation can be included in the ML model performance data, instead of including the data set, to reduce the size of the notification.

4.	NWDAF1 containing MTLF decides whether update of the trained ML model is needed, based on the notifications received in step 3a-3b. NWDAF1 containing MTLF may, if supports, use the ML model performance data in the notifications as input data to perform analytics on the trained ML model performance, and decide whether or not to update the trained ML model based on the statistics and/or predictions of the ML model performance. The information in ML model performance statistics and predictions is shown in Table 6.32.2-2 and Table 6.32.2-3.

Table 6.32.2-2: ML model performance statistics

| Information | Description |
| --- | --- |
| Analytics ID | The Analytics ID(s) associated with the trained ML Model(s). |
| Analytics deviation | Deviation of the predictions using the trained ML model from the actual network data |
| ML model accuracy | Accuracy of the analytics using the trained ML model |
| Spatial validity | Area where the trained ML Model(s) performance analytics applies. |
| Validity period | Validity period for the trained ML Model(s) performance analytics. |

Table 6.32.2-3: ML model performance predictions

| Information | Description |
| --- | --- |
| Analytics ID | The Analytics ID(s) associated with the trained ML Model(s). |
| Analytics deviation | Deviation of the predictions using the trained ML model from the actual network data |
| ML model accuracy | Accuracy of the analytics using the trained ML model |
| Spatial validity | Area where the trained ML Model(s) performance analytics applies. |
| Validity period | Validity period for the trained ML Model(s) performance analytics. |
| Confidence | Confidence of this prediction |

NOTE 3:	Here it is assumed that the NWDAF1 containing MTLF is capable of performing analytics (statistics and/or predictions) on ML model performance, based on the ML model performance data in the notifications from the NWDAF(s) containing AnLF, though the NWDAF1 containing MTLF does not support network analytics as described in TS 23.288 [5].

If update of the trained ML model is needed, NWDAF1 containing MTLF retrains the ML model using the data set (if provided) in the Nnwdaf_MLModelPerformance_Notify, or using the data collected from 5GC NF(s), OAM and/or UE application(s).

5a-5c.	NWDAF1 containing MTLF notifies NWDAF2 containing AnLF, NWDAF3 containing AnLF and NWDAF4 containing AnLF with the updated trained ML Model information by invoking Nnwdaf_MLModelProvision_Notify service operation, as described in clause 6.2A.1 of TS 23.288 [5].

NOTE 4:	Here without step 2 and step 3 performed towards/by NWDAF4 containing AnLF, NWDAF1 containing MTLF also updates the trained ML model towards NWDAF4 containing AnLF based on the subscriptions in step 1.

6.32.3	Impacts on Existing Nodes and Functionality

NWDAF containing MTLF:

-	Subscribes to notifications of the trained ML model performance feedback information.

-	Uses the ML model performance data in the notifications as input data to perform analytics on the trained ML model performance, and decide whether or not to update the trained ML model based on the statistics and/or predictions of the ML model performance.

NWDAF containing AnLF:

-	Provides notifications of the trained ML model performance feedback information to the NWDAF containing MTLF.

## 6.33	Solution #33: Improving correctness of NWDAF analytics by providing correction information

## 6.33.1	Description

This solution is proposed to address Key Issue #1: how to improve correctness of NWDAF analytics.

Analytics consumer may not use the analytics as soon as it receives analytics from NWDAF, while it may use the analytics only when it needs to make a decision, or the analytics consumer may use the analytics for a period of time and make several decisions during this time. In this case, there is a time period between the analytics received and the analytics actually used by the analytics consumer. Meanwhile, the NWDAF may improve the accuracy of the analytics or find the provided analytics is incorrect due to various factors (e.g., update of ML model, reselection of ML model, parameter adjustment, more data input, etc.) and the improved analytics may have positive impact to consumer's decision. Therefore, it is useful that NWDAF can provide correction information for provided analytics to the consumer (e.g., information to update provided analytics, notification of stopping using provided analytics, incorrect content of provided analytics, etc.).

As described in TS 23.288[5], analytics consumer may provide preferred level of accuracy and output strategy, which means accuracy variation within a certain range may not influence some consumers, therefore, different analytics consumer should determine whether it receives the correction information and the time period during which the correction information can be provided. To reduce signaling load, the frequency and condition of providing correction information should also be designated by analytics consumer, for example, the consumer can give a correction threshold, requiring NWDAF to provide correction information when NWDAF finds the accuracy improved over the threshold.

This solution proposes that consumers may provide requirement on correction information in subscription, and NWDAF maintains provided analytics and provides correction information for the latest provided analytics to the consumer.

In this solution, the following functionalities are proposed to take into consideration:

-	Analytics consumer provides following information in subscription if it requires to receive correction information:

  - -	Correction time period: defines the time interval during which the correction information can be accepted by the consumer. It is a relative time interval as the gap with respect to analytics is provided.

  - -	Condition of providing correction information: defines the condition when the correction information can be provided to the consumer, e.g., correction threshold (when NWDAF finds the accuracy difference between new analytics and provided analytics exceeding the specified threshold);

  - -	Preferred correction information form: defines the information form in which the consumer requires to receive. It can be set as following values:

    - -	an incorrectness notification: used to notify the consumer provided analytics is incorrect;

    - -	an updated analytics: the NWDAF can provide an updated analytics to replace provided analytics;

    - -	an error value: NWDAF can provide a delta compared to the provided analytics.

-	NWDAF maintains provided analytics during correction time period.

-	NWDAF provides correction information for the latest provided analytics to consumer according to the preferred correction information form in subscription.

### 6.33.2	Procedures

Figure 6.33.2-1 illustrates the procedure for NWDAF providing correction information.

Figure 6.33.2-1: Procedures for improving correctness of NWDAF analytics by providing correction information

1.	The NF (analytics consumer) sends Nnwdaf_AnalyticsSubscription_Subscribe/ Nnwdaf_AnalyticsInfo_Request. This service operation is enhanced with providing more parameters which include correction time period, condition of providing correction information (e.g., correction threshold), preferred correction information form (e.g., incorrectness notification, updated analytics, …), if the NF requires the correction information for provided analytics.

2.	The NWDAF collects input data and performs analytics as the procedures in TS 23.288[5].

3.	The NWDAF sends the analytics output to NF as the procedures in TS 23.288[5].

4.	The maintenance of provided analytics will be triggered depending on various factors (e.g., model updated, reselection of ML model, parameter adjustment, more input data, etc.).

5.	If step 4 occurs within correction time period, NWDAF maintains provided analytics.

If condition of providing correction information is met, the NWDAF generates the correction information in preferred correction information form as the consumer required.

6.	The NWDAF sends the correction information for the latest provided analytics to NF.

### 6.33.3	Impacts on Existing Nodes and Functionality

NWDAF:

-	Support on maintaining provided analytics.

-	Support on generating correction information for the latest provided analytics.

Analytics consumer:

-	Support on providing requirement related to correction information in subscription information.

## 6.34	Solution #34: Enhancing the accuracy of NWDAF Analytics

### 6.34.1	Description

This solution addresses aspects of key issue #1 on how to improve correctness of NWDAF analytics.

When NWDAF predicts something for the future, it does not monitor/collect information about the occurrence of the actual event, so it cannot compare the true observed event with its corresponding previous predictions.

NOTE:	The NWDAF in this solution refers to an NWDAF that and exposes at least the analytics services, i.e., Nnwdaf_AnalyticsSubscription and/or Nnwdaf_AnalyticsInfo, therefore an NWDAF containing AnLF applies.

Similarly, consumers requesting an analytics of type "prediction" from an NWDAF, are provided only analytics output/prediction with a confidence estimated by NWDAF implementation logic. An analytics consumer does not retrieve an accuracy report related to the prediction or related to earlier predictions done for the same analytics type and model used (i.e., an accuracy indication derived from comparing the actual outcome with a previous predicted value).

This solution proposes to enable the NWDAF to determine the actual outcome of a prediction, so that it can either:

-	Report in an accuracy report, the outcome to the consumer so the consumer can compare the outcome with the previously provided prediction, or

-	Compare the outcome with the previous prediction to generate an accuracy report which the NWDAF provides to the consumer.

The solution comprises the following steps:

-	NWDAF (continues to) collects new data after a prediction has been made, so that it can learn about actual outcomes, compare them with the predicted data, and generate an accuracy report. The accuracy report comprises the accuracy for a given analytics subscription or for a given Analytics ID (e.g., average over all subscriptions for that Analytics ID). The accuracy report may provide the actual outcomes, or information on how the outcomes deviate from the predicted values. When aggregated accuracy is provided (e.g., for all subscriptions to an analytics ID), the accuracy reports may be provided as a function of the prediction confidence level (i.e., probability assertion). If an NWDAF makes a prediction with low confidence, one should expect lower accuracy compared to when the NWDAF makes a prediction with high confidence. This is reflected in the accuracy reports.

Editor's note:	The details of the format of the accuracy report are FFS.

-	NWDAF Analytics services are enhanced to enable an analytics consumer to collect a detailed accuracy report from NWDAF (e.g. how well the NWDAF performed for a specific Analytics ID, using a specific model, for a specific area/slice(group of)UE, in a given time window).

-	NWDAF registers its measured accuracy (per Analytics ID) in its NF profile in NRF/UDM, so that analytics consumers can use this information when selecting the NWDAF instance/set.

-	NWDAF uses the accuracy information to further fine-tune the model it is using (e.g., request model re-training, adjust data sources for the model training and the analytics generation).

Editor's note:	Whether and how the accuracy report from AnLF is provided to MTLF to improve the correctness of analytics is FFS.

-	The analytics consumer uses the accuracy report provided by NWDAF to manage the subscription (e.g. if accuracy is low the analytics consumer may terminate the subscription, adjust subscription parameters, or change the NWDAF, etc..) and/or to take the actual accuracy into account into local decision making (e.g., adjust the weight of the analytics reports in the decision making compared to other inputs).

### 6.34.2	Procedures

### Figure 6.34.2-1 illustrates the procedure for measuring and reporting accuracy of NWDAF analytics.

Figure 6.34.2-1: Procedure for measuring and reporting accuracy of NWDAF analytics

1.	The (consumer) NF sends Nnwdaf_AnalyticsSubscription_Subscribe request with a flag indicating that it is interested in receiving an accuracy report along with the analytics (prediction) report. The NFc also provides a callback URI to receive the accuracy report. The NFc may also send separate processing and filtering instructions for the accuracy report(s) (e.g., to club or filter the accuracy reports). The NFc may also send instructions to NWDAF on how to handle the subscription after having generating accuracy report. For example, the instructions may specify that if three consecutive predictions are wrong, the NWDAF subscription must be terminated, and a termination notification will be sent to the NFc.

2.	The NWDAF generates the predictions and provides them to the NFc.

3.	The NWDAF collects the accuracy data, i.e., the actual measured data observed at the time for which the prediction had been made. Accuracy data can be collected from different data sources, not necessarily limited to the data sources used as input data sources for generating the analytics prediction.

4.	An accuracy report is sent to the consumer after the events have occurred that were needed for the verification of the predictions. The accuracy report may be sent in a separate notification from NWDAF to NFc or it can be clubbed with the next analytics notification sending predictions of future events. The frequency of the accuracy notification may be controlled via the filtering/reporting instruction(s) given in the subscribe request in step 1.

5.	(optional) If, in step 1, an instruction was received on how to handle the subscription after having terminated the accuracy report, the NWDAF executes such instruction. For example, NWDAF terminates the subscription if three consecutive accuracy reports are below a given threshold (e.g., "no match") by sending a termination request.

### 6.34.3	Impacts on services, entities and interfaces

The solution has the following impacts:

NWDAF:

-	Keeps collecting data after a prediction is made from corresponding data provider(s) to calculate the accuracy of such prediction.

-	generates accuracy reports to be sent to analytics consumers, upon their request.

-	NWDAF registers its measured accuracy (per Analytics ID) in its NF profile in NRF/UDM.

Nnwdaf_AnalyticsSubscription service:

-	_subscribe service operation includes an Accuracy Report Required flag and additional filter information related

-	_notify service operation includes the Accuracy Report.

## 6.35	Solution #35: Improve model training and provisioning exploiting sub-areas with similar statistical properties

### 6.35.1	Description

This solution addresses aspects of key issue #1 on how to improve correctness of NWDAF analytics.

Knowing the statistical properties of a data set used in AI/ML is important to understand the behavior of an ML model. ML models trained on a specific training data set can be employed on data with similar statistics without experiencing performance degradation. Different network areas may exhibit similar context, (e.g., geographical circumstances, network deployment, etc.), which leads to similar data statistics (data statistics refer to data distribution, i.e., the information on values - or intervals - of the data such as network load, interference, how frequent some data values occur, etc.). So, an AI/ML model trained using data from one network area could be utilized in a different network area that exhibits similar data statistics.

Furthermore, if multiple AI/ML models are employed in one area, even employed for different analytics and/or used by different analytics functions, information on changes in that area can be used by all analytics functions to, e.g., update their models given that change in the analytics context.

This solution is based on the ability to divide and characterize the network into sub-areas based on the environment statistical properties instead of a purely geographical sub-division. This statistical characterization of network sub-areas is then used to improve the quality of ML model training which should in turn improve the correctness of the corresponding NWDAF Analytics.

This solution proposes the introduction of the "Area monitoring analytics service", which provides AOI partitioning based on environment statistical properties and takes as parameters in the service request (see Table 6.35.1-1) the area of interest (AOI) to be analyzed, the area granularity attribute and other analytics parameters (e.g., reporting_threshold indicating the threshold for receiving the reporting, anomaly_description indicating the desired reporting content). Environment statistical properties aim at representing how an area is used, and can include density of UEs per area, UE behavior in the area such as UE mobility model and UE communication model, etc.

The resulting AOI partitioning into sub-areas with associated sub-area types detected (i.e., dividing the AOI in multiple sub-area types and associating to each of them the respective network elements) along with the description of their statistical properties are stored in the Analytics Area Type Properties Function (AATPF), that allows the tracking of the discovered sub-area types and supports their management. The Sub-Area type ID is an identifier to be assigned to a sub-area and its properties description (area size specified by area granularity attribute or according to the data distribution) based on its data statistics.

NOTE:	The AATPF can be a standalone function or co-located with NWDAF.

The Area monitoring service can be consumed by OAM. Furthermore, an NWDAF containing AnLF can be a consumer too, to know the sub-area types in order for it to re-use and select appropriate ML models.

Table 6.35.1-1 lists the parameters to be included in the service request of Area monitoring analytics.

Table 6.35.1-1: Parameters in the service request of Area monitoring analytics.

| Parameter | Description |
| --- | --- |
| AOI | Area of interest (AOI) to be analyzed/monitored. |
| Sub-area type (0..max) | (optional) Sub-area type(s) to be analyzed/monitored. |
| Area granularity | Defines the size of the sub-areas that the AOI should be partitioned into. The attribute can, e.g., be provided as an absolute value (e.g., in km2), as a rough indication (e.g. as building/ street/ block/ district/ city/ state), or as resolution indication such as low/mid/high (e.g., “high” indicating that all distinguishable sub-areas shall be reported, “low” indicating that only few sub-areas with most extreme data statistics shall be reported). In case area granularity is not specified by the consumer, the AOI is partitioned according to the data distribution detected in the AOI (i.e., the resulting sub-areas may have different sizes depending on the data statistics). |
| Reporting threshold | Indicates the threshold for receiving the reporting. |
| Anomaly description | Indicates the desired reporting content, e.g., "full", "partial", or "none" description of the detected anomaly. See also Table 6.35.1-3. |

Table 6.35.1-2 lists the input data and the associated source of Area monitoring analytics.

Table 6.35.1-2: Input data and the associated source of Area monitoring analytics.

| Information | Source |
| --- | --- |
| Application ID | AF |
| IP filter information | AF |
| Locations of Application | AF/NEF |
| Service Experience | AF |
| QoE metrics | UE (via AF) |
| Timestamp | AF |
| NF load | NRF |
| NE resource usage | OAM |
| Traffic usage report | UPF |
| Status, load and performance information | OAM |
| Number of UEs in sub-area defined by area-granularity or by data statistics | AMF |
| UE locations | AMF |
| UE access behaviour and location trends | AMF |
| Expected UE behaviour parameters | AF |
| UE communication | UPF, AF |
| UE session behaviour and communication trends | SMF |
| Expected UE behaviour related network data analytics | NWDAF |
| UE related analytics as UE mobility analytics and/or UE communication analytics | NWDAF |
| Expected environment properties | NWDAF, service consumer |

Table 6.35.1-3 lists the output data of Area monitoring analytics.

Table 6.35.1-3: Output data of Area monitoring analytics.

| Information | Description |
| --- | --- |
| Sub-area ID (1..max) | List of sub-area identifiers. May be the complete list of determined sub-areas in the AOI (e.g. when initially providing the list to the service consumer) or may only comprise those sub-areas in the AOI affected by the environment properties change. |
| > Sub area coordinates | (optional) Specifies the geographical information of the sub area. See NOTE 1. |
| > Sub-area type ID | Uniquely identifies the sub-area and its environment properties. This information can be used to determine sub-areas with the same sub-area type. When temporary flag is set to False this shall be set to the new sub-area type ID if changed compared to the previous notification. |
| > Expected environment properties | Optional parameter providing information about the expected environment properties of this sub-area type ID. Properties can include density of UEs per area, UE mobility model in the area, and UE communication model in the area, etc. See NOTE 2. |
| > Anomaly | Optional parameter providing information about an observed change in the environment properties. See NOTE 2. |
| >> Anomaly level | Type and level of the anomaly in the sub-area detected, e.g., minor change, trend, pattern, etc. It shall be set to "initial" when this analytics report is providing information about the initial sub-area characterization. |
| >> Anomaly description | "Full" or "partial" description of the properties change detected. If "Full" is selected by the consumer in the service request, the complete properties description shall be returned, otherwise only the Delta compared with the usual environment properties shall be returned. The description also includes information about the anomaly, e.g., estimated duration, periodicity, new or recurrent anomaly, etc.. |
| >> Temporary | Indicates if the anomaly is temporary or permanent. |
| >> Time | Indicates since when the anomaly has been detected. |
| > Confidence | Indicates the confidence about the sub-area environment properties identified (if anomaly level is "initial") or confidence about the change observed. |
| NOTE 1: This information may be skipped if the sub area coordinates have not changed compared to the previous notification and shall be provided otherwise. NOTE 2: Either "expected environment properties" or "anomaly" parameter is returned. |  |

### 6.35.2	Procedures

6.35.2.1	Detecting and managing sub-areas using the Area Monitoring analytics service

In this procedure the NWDAF service consumer subscribes to the Analytics ID "area monitoring". The NWDAF analyzes the expected environment properties for a set of sub-areas within the AOI(s) provided in the analytics request/subscription.

Figure 6.35.2.1-1: Procedure for detecting and managing sub-areas using the Area Monitoring analytics service

Pre-condition: The Analytics Area Type Properties Function (AATPF) stores the mapping of AOI in sub-areas along with their Sub-Area type IDs assigned and properties descriptions.

1.	NWDAF service consumer (NF or OAM) subscribes to analytics information. It sets, among others input parameters (as detailed in TS 23.288 [5], clause 6.1), the Analytics ID to the "Area Monitoring" analytics service ID, the (list of) AOI(s) and, optionally, the area granularity attribute. In addition, the service consumer may provide a reporting threshold indicating for which level of environmental properties changes a notification shall be returned and an anomaly description indicating if it is interested to receive the "full" description of the environment properties or the "delta" or "none".

2-3.	NWDAF subscribes to the AATPF to receive the AOI partitioning, Sub-Area type IDs and their properties description using the Naatpf_AreaTypeInfo_Subscription service and receiving as response the Naatpf_AreaTypeInfo_Notify service operation.

4-5.	NWDAF subscribes to the data sources required for the analytics services.

6.	If "area granularity" is provided (see table 6.35.1-1), the classification of each sub-area of the AOI is performed by the NWDAF and their properties descriptions are generated. Otherwise, if "area granularity" is not provided, the NWDAF determines the sub-areas based on the data statistics. NWDAF classifies the sub-areas periodically and/or sporadically (e.g., based on internal/external triggers) based on new data collected.

7-9.	(conditional) Whenever new environment properties are discovered, NWDAF sends to the AATPF, using the Naatpf_RegisterAreaType_Register service, the AOI partitioning, and the sub-areas properties descriptions generated at step 6. The AATPF assigns new Sub-Area type IDs to the new environment properties discovered and stores the updated AOI partitioning along with the properties’ description. In step 9, the AATPF informs the NWDAF that the operation has been concluded and provides the Sub-Area type IDs associated with the sub-areas detected. The AATPF may store the "Expected environment properties" for the monitored AOI(s) in some other entity, e.g., the ADRF. Such copy may, e.g., be used by another NWDAF instance also providing the "Area Monitoring" service for the same/overlapping AOI(s), to recover the information from the ADRF in case the NWDAF instance is (re-) started, and/or to allow the NWDAF to compare the current properties with an older version of the "expected environment properties".

10.	NWDAF periodically and/or based on internal/external triggers, keeps analysing the AOI. If a change in the environment properties is detected (e.g., the delta of the current environment properties is above a threshold compared to the earlier properties description), the following steps 11 to 19 are executed. The NWDAF may also utilize earlier versions of the "expected environment properties" available locally or in the ADRF to determine whether abnormal environment properties are observed, or, e.g., whether such environment properties have already been observed in the past (e.g., periodically every week).

11.	The NWDAF notifies the consumer that an anomaly in a sub-area has been detected. The NWDAF sets the flag temporary to True to inform the service consumer that the sub-area could come back to expected environment properties.

12.	The NWDAF monitors the sub-area(s) where the abnormal environment properties have been detected.

13.	(optional) If the anomaly is still present, the NWDAF periodically informs the analytics consumer. The notification may include an indication about the type of the change, e.g.:

-	"trend": indicates that with respect to previous properties the delta might be small (e.g., might be below the reporting threshold), but comparing with older data a trend can be observed (e.g., the area is getting more busy).

-	"pattern": indicates a repeating change of the environment properties (e.g., observed difference between working days, Saturdays and Sundays/holidays).

-	"abnormal/exception": change above reporting threshold that is not a "trend" or "pattern".

The notification may further include the time value since when this change has been observed, as well as additional information about the change, e.g., the expected duration, periodicity, estimated future increase for certain values, etc. Furthermore, the notification includes the impacted sub-areas, their Sub-Area Type IDs, and the anomaly description if requested.

14.	The NWDAF utilizing the Naatpf_RegisterAreaType_Update service, sends to the AATRF the abnormal properties descriptions generated with the flag temporary set to True.

15.	The AATPF acknowledges to the NWDAF that the request has been received.

16.	(conditional) If environment properties return back to the expected properties or timer T expires, the NWDAF sends an area type update to the AATPF setting the temporary flag to False.

17.	(conditional) If the AOI environment properties still differ from the expected ones, the AATPF updates the Sub-Area type IDs and their properties description.

18.	The AATPF informs the NWDAF about the new Sub-Area Type ID assigned to the sub-area.

19.	The NWDAF notifies the analytics service consumer about a permanent deviation from the previous "expected environment properties", i.e., the temporary flag is set to False. The notification may include an indication about the type of the change, see step 13. Alternatively, if the environment properties are back to the earlier state, the NWDAF notifies the service consumer about the end of the abnormal situation.

6.35.2.2	Usage of sub-areas to improve ML model provisioning

Figure 6.35.2.2-1: Procedure for usage of sub-areas to improve ML model provisioning

Precondition: NWDAFs update their profiles stored at the NRF to include the Area types managed. AATPF stores the mapping <AOI, Area type ID> and respective properties description. AATPF is the function deputed to assign Area type IDs to sub-areas based on their data properties description. It avoids the usage of multiple Area type IDs for sub-areas with similar data properties descriptions as well as the usage of the same Area type ID for different data properties descriptions. NWDAF (MTLF) registers for each AI/ML model the Area type for which it has been trained.

1.	NWDAF service consumers send a discovery request for NWDAF to NRF specifying the AOIs and the Analytics ID. If the NWDAF service consumer is aware about the Area type for which it is interested, it can specify the Area type in the discovery request.

2.	The NRF returns the list of NWDAFs matching the parameters specified in the discovery request. Furthermore, the NRF also returns the Area types supported by each discovered NWDAF.

3.	NWDAF service consumers subscribe to selected NWDAF(s) serving the AOIs specifying the Area type if known. In case the Area type is not provided (e.g., not known), the analytics service is requested for all the Area types included in the AOI.

4.	NWDAFs request to the NWDAF (MTLF) a trained AI/ML model specifying among others the novel Area type attribute. In this way, AI/ML models trained for the same area type, but different geographical location, can be used to produce analytics in the specified AOI.

5.	NWDAF (MTLF) provides to different NWDAFs (managing different AOIs) the required trained AI/ML models. For NWDAFs requesting a model for the same Analytics ID in sub-areas assigned to the same Area type ID, NWDAF (MTLF) deploys the same AI/ML model trained for that Analytics ID for the specific Area type ID. This allows to use an existing AI/ML model from another AOI but trained for the same Area type ID even when there is no specific AI/ML model trained for that AOI. In the case that there is no availability of the AI/ML model trained for requested Area type ID, the NWDAF (MTLF) may perform the model training for missing Area type ID by collecting the data from multiple AOI with the same Area type or with similar data properties in order to have a richer training dataset.

6.	The NWDAF(s) run the AI/ML models to produce the requested analytics.

7.	NWDAF(s) notify the NWDAF Service Consumer(s) with the analytics reports requested.

### 6.35.3	Impacts on services, entities and interfaces

The solution has the following impacts:

-	Introduction of the "Area monitoring analytics service" exposed by the Analytics Area Type Properties Function (AATPF.

NWDAF

-	Support for additional parameters in the Nnwdaf_AnalyticsSubscription_Notify service operation.

NRF

-	Support for additional parameters in the Nnrf_discovery service operations.

## 6.36	Solution #36: Enhanced provisioning of ML model based on information about how inference will be executed

### 6.36.1	Description

This solution addresses aspects of key issue #1 on how to improve correctness of NWDAF analytics.

When an NWDAF acting as service consumer invokes the Nnwdaf_MLModelProvision_Subscribe service of an NWDAF containing an MTLF, the MTLF may perform one of the following:

a)	Select an existing trained ML Model to be provisioned.

b)	Trigger further training of an existing trained ML model before it is provisioned.

c)	Generate a new ML model to be trained and provisioned.

In cases b) and c), the NWDAF containing MTLF may initiate data collection from NFs, AF, DCCF, or OAM, required for the training of the ML model. Therefore, the decision of the MTLF among options a, b, and c can:

i.	lead to very different levels of computational load for the NWDAF containing MTLF.

ii.	increase the networking load on the NWDAF containing MTLF as well as on other Network Functions (NFs), while this increase might highly vary depending on how many data sources are used.

The decision of the MTLF among options a, b, and c is currently (i.e., according to Rel.17 specifications) taken by the MTLF based on local, unspecified logic. Based on the information currently provided in the Nnwdaf_MLModelProvision_Subscribe request, even if the "required accuracy" is provided as well, the MTLF may decide in favour of an option that leads to high computational and networking overhead without any real benefit (i.e., not really increasing the achieved accuracy during the inference phase at the NWDAF containing AnLF) compared to another option, which would put less load on the system.

More concretely, if an NWDAF service consumer subscribes to an NWDAF containing MTLF for an ML model for a specific type of analytics, the MTLF may decide that the best option is to perform further training of an existing model (i.e., option b) or create a new model because it is expected to lead to higher accuracy (i.e., option c). However, it can happen that the accuracy achieved when the ML model is later used by the NWDAF containing AnLF is not higher than the accuracy that would be achieved by an already trained (i.e., option a). Further, based on how the NWDAF containing AnLF is using the ML model, the same accuracy might be achieved by a more lightweight version of the ML model.

According to TS 23.288 clause 6.2.1, an NWDAF containing AnLF, may:

-	use only a subset of the parameters and/or data sources specified as allowed input parameter for the analytics ID.

-	use only data from a reduced extend (e.g., duration, scope) of the data collection.

-	request different sampling ratios and/or partitioning criteria to the data source.

For example, an NWDAF containing AnLF, might determine to only use UE mobility information collected from AMF, but not use UE location information provided by OAM. Then, the NWDAF containing AnLF could use a potentially more lightweight ML model that was trained only on AMF data, resulting in the same accuracy of the analytics as when using an ML model that was trained on data collected from both AMF and OAM.

This solution proposes to exchange and use information related to how inference will actually take place at the NWDAF analytics consumer once the ML model has been received. The solution also proposes enhancing and optimizing the ML model selection and provisioning (i.e., selection of trained model, trigger further training of an existing ML model, or generate a new model to be trained) performed by NWDAF containing MTLF.

In this solution, the ML model provisioning request includes inputs about the data used for inference, such as the actual data sources that will be used for inference, the granularity of input data that will be used for inference, and the target environment capacity requirements. Similarly, to assist the AnLF in selecting the appropriate ML model for inference, the ML model provisioning response includes information about the data (parameters and data sources) that have been actually used for the training of the ML model. The new parameters are described in table 6.36.1-1.

Table 6.36.1-1: Description of new parameters proposed in this solution

| Parameter | Description |
| --- | --- |
| Data used for inference | This parameter provides information on the actual data sources that will be/have been used for generating the analytics (i.e., for the inference at the NWDAF containing AnLF). For the data sources to be used (e.g., AMF and OAM), further information can be provided in order to indicate specific instances or sets that will be used during inference, or specific parameters that will be collected from those data sources (e.g., UE location from AF and Application ID from AF, while all parameters except UE location from the AMF). |
| > Granularity of input data | The granularity of input data that will be used for inference may be expressed as minimum sampling ratio, maximum time interval, and/or total number of input values. |
| Data used for training | This parameter provides information on the actual data and data sources that have been used for the (re-)training of the ML model. It also provides information on the granularity of the used input data, the extend (e.g., duration, scope) of the data, the sampling ratio, and the partitioning criteria, if applicable. |
| > Granularity of input data | The granularity of input data that will be used for inference may be expressed as minimum sampling ratio, maximum time interval, and/or total number of input values. |
| Target environment capacity requirements | The information about the target environment capacity requirements includes, e.g., the computational capacity and the memory that will be available when performing inference with this model. |

### 6.36.2	Procedures

### Figure 6.36.2-1 illustrates the procedure for enhancing the provisioning of an ML model based on information about how inference will be executed. The procedure includes the request of analytics data from an NF service consumer, which may be the trigger for the provisioning of an ML model from the NWDAF containing MTLF to the NWDAF that acts as an NF service consumer for ML model provisioning (i.e., NWDAF containing AnLF).

Figure 6.36.2-1: Procedure for enhanced provisioning of ML model based on information about how inference will be executed.

1.	The NF consumer subscribes an Analytics ID to an NWDAF containing AnLF, using the Nnwdaf_AnalyticsSubscription_Subscribe service operation.

2.	The NWDAF containing AnLF requests an NWDAF containing MTLF an ML model that can be used to support the Analytics ID requested in step 1, either because it has no ML model for this purpose or because the ML model(s) it has may not perform sufficiently well. Step 2 may be triggered by step 1, but it may also have been triggered by a different trigger before the execution of step 1. The request for the ML model is performed by sending an Nnwdaf_MLModelProvision_Subscribe request to the NWDAF containing MTLF with the addition of the parameters "data used for inference", the "input data granularity", the "target environment capacity requirements" and the "requested accuracy".

3.	The NWDAF containing MTLF selects an existing trained ML model or creates a new ML model to be provisioned and determines the level of further training of the ML model, based on the "data used for inference", the "input data granularity", the "target environment capacity requirements" and the "requested accuracy" provided in step 2.

4.	The NWDAF containing MTLF determines the training data and data sources required for (re-)training the ML model determined in step 3 and collects the data from the determined data source(s). The NWDAF (re-)trains the ML model using the collected data.

5.	The NWDAF containing MTLF provides to the NWDAF containing AnLF the ML model using the Nnwdaf_MLModelProvision_Subscribe response, with the additional parameter "data used for training" indicating the data and data sources including the "input data granularity" used for the ML model training, .

6.	The NWDAF containing AnLF uses the information received in step 5 to determine the ML model to use in order to compute the analytics that were requested in step 1. Then it executes the inference, e.g., based on the received ML model,

7.	The NWDAF containing AnLF sends to the NF consumer notifications with the computed analytics for the requested Analytics ID, adding information about which data and data sources have been used for inference. This information can be useful in order to determine the level of trust that the analytics consumer can have in the results received.

8.	The NWDAF containing AnLF determines that a data source to fulfill the Analytics subscription received in Step 1 has changed.

9.	The NWDAF containing AnLF determines that it no longer needs to receive notifications about availability of ML models associated to the previous input data sources.

10.	[Conditional to step 9.] The NWDAF containing AnLF uses the Nnwdaf_AnalyticsSubscription_Unsubscribe service operation to quit the subscription to ML models.

If a new ML model is required, the procedure restarts from step 2.

### 6.36.3	Impacts on services, entities, and interfaces

The solution has the following impacts:

-	Additional parameters ("data used for inference", "input data granularity", "target environment capacity requirements" and "requested accuracy") in the Nnwdaf_MLModelProvision_Subscribe and Nnwdaf_MLModelProvision_Notify service operations.

-	Additional parameters (data and data sources used for inference) in the Nnwdaf_AnalyticsSubscription_Notify service operation.

6.37	Solution #37: Analytics Exchange in Home routed roaming case

6.37.1	Description

This solution is for the Key Issue#3: Data and analytics exchange in roaming case.

Some principles for the data and analytics exchange in roaming case are proposed as follows:

-	No raw data exchange is allowed between the H-PLMN and V-PLMN considering series of factors e.g. data privacy and security, signalling pressure on the SEPP;

-	Analytics exchange may be allowed between the H-PLMN and V-PLMN:

-	What direction of the analytics exchange and between the H-PLMN and V-PLMN and which Analytics ID can be exchanged should be dependent on specific use cases. For example, this solution proposes that the V-NWDAF provides the service experience analytics to H-PCF to help determine the QoS parameters for an application; in Solution#11, the H-NWDAF can provide the slice Load level information to V-NWDAF to help V-AMF for network slice selection;

-	User consent should be checked before the analytics exchange.

Editor's note:	It is FFS on how the user consent is checked in roaming scenario and the cooperation with SA3 is needed.

Editor's note:	It is FFS if the V-PLMN is aware of user consent.

Editor's note:	It Is FFS to which extent the VPLMN can provide meaningful analytics for a new inbound roamer without historical data about the roaming user and with only limited access to input data sources in the HPLMN. Depending on the results, allowing collection of raw data from VPLMN may be indispensable.

Editor's note:	It Is FFS whether raw data aggregation can be an alternative to analytics to reduce signalling load.

Because this solution assumes that no raw data exchange is allowed, so the H-NWDAF and the V-NWDAF will respectively take the responsibility for data analytics in their own network and then further exchange the (sub-)analytics with each other.

To prevent the access of the raw data, this solution proposes that only the Nnwdaf_Analytics services are allowed between the H-NWDAF and the V-NWDAF.

Take the use case i.e. "PCF uses service experience as input to calculate and update the authorized QoS for an application" as defined in clause 6.1.1.3 of TS 23.503 [4] as an example, this solution proposes that the H-PCF can adjust QoS parameters for the application in home routed roaming case based on the service experience analytics.

In details, for an Application ID, the V-NWDAF and the H-NWDAF firstly derive the service experience analytics in visited network and the service experience analytics in home network, respectively. Based on the H-PCF's service experience analytics subscription, the H-NWDAF further subscribes the service experience analytics in visited network for the application to V-NWDAF, then the H-NWDAF derives the (E2E) service experience analytics based on the service experience analytics in visited network and the service experience analytics in home network and provides it to H-PCF. Finally, H-PCF may use the E2E service experience as input to calculate and update the authorized QoS for the application in HR roaming case.

6.37.2	Procedures

6.37.2.1	Procedure for the service experience analytics subscription from H-NWDAF to V-NWDAF

Figure 6.37.2.1-1: Procedure for the service experience analytics subscription from H-NWDAF to V-NWDAF

1.	PCF firstly determines one or more UEs for an application are in home routed roaming case, then the PCF in home network subscribes service experience analytics of an application for these UEs to the H-NWDAF by triggering the Nnwdaf_AnalyticsSubscription_Subscribe service operation. The parameters included in the service operation are defined in clause 6.4.1, TS 23.288 [5]. In addition, optionally, a HR roaming indicator and the V-PLMN ID should be also included in the service operation to help H-NWDAF know that the UEs are in HR roaming case. Alternatively, the H-NWDAF can also query the UDM to know that the UEs are in HR roaming case.

2.	H-NWDAF discovers the V-NWDAF by using the V-PLMN ID via NRF and then subscribes the service experience analytics in the visited network to the V-NWDAF by triggering the Nnwdaf_AnalyticsSubscription_Subscribe service operation.

3.	V-NWDAF collects data in the visited network for the UE by using the procedures as defined in clause 6.2.2, TS 23.288 [5].

4.	V-NWDAF derives the service experience analytics in the visited network based on the collected data in visited network.

5.	V-NWDAF provides the service experience analytics in the visited network to H-NWDAF by triggering the Nnwdaf_AnalyticsSubscription_Notify service operation.

6.	H-NWDAF collects data in the home network for the UE by using the procedures as defined in clause 6.2.2, TS 23.288 [5]. In the home routed remaining case, the AF is located in the home network, so it is the H-NWDAF's responsibility to collect the service data from AF.

7.	H-NWDAF derives the service experience analytics in the home network based on the collected data in home network.

8.	H-NWDAF derives the E2E service experience analytics based on the service experience analytics in the visited network and the service experience analytics in the home network.

9.	H-NWDAF provides the service experience analytics for these UEs to the PCF in home network.

10.	The PCF may adjust the QoS parameters for the application based on the E2E service experience analytics.

6.37.3	Impacts on Existing Nodes and Functionality

H-NWDAF:

-	Supports to accept the service experience analytics subscription request for UE in home routed roaming case from H-PCF.

-	Supports to subscribe service experience analytics in the visited network from V-NWDAF;

-	Support to derive the E2E service experience analytics based on the service experience analytics in the visited network and the service experience analytics in the home network.

V-NWDAF:

-	Supports to accept the service experience analytics in the visited network subscription from H-NWDAF;

6.38	Solution #38: Interactions between VPLMN and HPLMN for restricted data collection and analytics retrieval

6.38.1	Description

6.38.1.1	General

Data or analytics exposed to a peer PLMN need to be restricted based on operator policies, regulatory requirements, roaming agreements, and user consent.

It is suggested that the NWDAF serves as central entry point for related requests and authorizes requests and restricts exposed data and analytical information. New services at the NWDAF allow to invoke those special authorization and restriction functionality.

The access to other services of the NWDAF, to DCCF services and to exposure services at other NFs from peer PLMNs can be restricted based on operator's policy by the NRF: The NRF will not providing related access tokens.

The NWDAF is well suited to handle analytics and data collection requests as it already performs comparable operations. The NWDAF also has existing capabilities for user consent checking. Using the NWDAF for data collection also enables usage of the DCCF.

6.38.1.2	HPLMN collecting data from VPLMN

A UE from HPLMN roams in a VPLMN network. NWDAF of HPLMN collects data from VPLMN NFs. This enables the NWDAF to collect more complete statistics about a UE or group of UEs by taking into account also data collected while the UE or group of UEs is roaming in some VPLMN. Data may for instance relate to locality, mobility and communication of the UEs while roaming in the VPLMN. Collected data may also enable the HPLMN to provide statistics taking into consideration the behavior of UEs subscribed at the HPLMN while roaming, which in turn enables a VPLMN to optimize the provided service for an inbound roaming UE.

The H-NWDAF from the HPLMN contacts the V-NWDAF of the VPLMN and asks it to retrieve data. The V-NWDAF checks if the HPLMN is authorised to request the data based on operator polices, interacts with different NFs in the VPLMN to retrieve the requested data, anonymizing the data if mandated by operator policies and/or user consent, and returns the requested information to the H-NWDAF. The V-NWDAF may also store the exposed data for subsequent audits and/or future use.

Based on operator's policy/Oauth2.0 security, NFs from HPLMN are only allowed to access a specific NWDAF_DataRetrieval service at V-NWDAF (service level protection is available). Other services (e.g., NF exposure services, DCCF data collection service) are restricted and the NRF in the VPLMN rejects related inquiries from the HPLMN and may redirect to the Nnwdaf_DataRetrieval service.

6.38.1.3	VPLMN consuming analytics generated by HPLMN

When UE1 enters a VPLMN network (i.e., it is roaming), the network does not have any historical data related to UE1 that it can use to create analytics (statistics and/or predictions). It can only start to collect data after the UE1 has entered the VLPMN, so the initial accuracy of the analytics reports would be rather low. However, HPLMN may have such data available for UE1 and can provide to the VPLMN NWDAF information related to UE1 that can help the VPLMN to optimize the service for the UE.

The HPLMN therefore creates an analytics profile of a UE subscribed at the HPLMN and may share it with VPLMNs restricting the contents considering regulatory requirements and operator policies. The access may be restricted to specific VPLMNs based on operator policy or user consent restricted specific VPLMNs, and parts of the UE analytics profile contents may only be accessible from specific VPLMNs, for instance to satisfy operator or regulatory policies or user consent restrictions.

The "VLPMN specific user consent" contains information on whether and which information about the UE may be shared with a VPLMN, whereby different filter information may be provided per VLPMN.

The "UE analytics profile" is generated by the NWDAF of the HPLMN (H-NWDAF) and stored in the ADRF of the HPLMN (H-ADRF).

The NWDAF of the VPLMN (V-NWDAF) can retrieve the UE analytics profile from the H-NWDAF for an inbound roaming UE and use it for determining analytics related to that UE for consumers NFs in the VPLMN. It may store the profile in the ADRF of the VPLMN (V-ADRF).

The UE analytics profile may contain:

o	UE Mobility patterns

o	UE Location patterns

o	Expected UE behavioral parameters

o	UE QoS / Congestion Experience patterns

o	UE Data usage patterns (e.g. indicating where a UE disperses most of its data volume and session transactions)

o	Data related to the UE that the V-NWDAF can use to generate its own analytics about the UE.

Based on operator's policy/Oauth2.0 security, NFs from VPLMN are only allowed to access a specific NWDAF_DataRetrieval service at H-NWDAF (service level protection is available). Other services (e.g., NF exposure services, DCCF data collection service) are restricted and the NRF in the HPLMN rejects related inquiries from the VPLMN and may redirect to the Nnwdaf_DataRetrieval service.

6.38.2	Procedures

6.38.2.1	HPLMN collecting data from VPLMN

Figure 6.38.2.1-1: HPLMN collecting data from VPLMN

1.	H-NWDAF of HPLMN discovers V-NWDAF of VPLMN. Based on operator's policy/Oauth2.0 security, H-NWDAF is allowed to access only a specific NWDAF_DataRetrieval service at V-NWDAF (service level protection is available). Other services (e.g., NF exposure services, DCCF data collection service) are restricted and the NRF in the VPLMN rejects related inquiries from the HPLMN and may redirect to the Nnwdaf_DataRetrieval service.

H-NWDAF requests data collection via V-NWDAF data retrieval service. It indicates as reporting target one or several UE(s), a UE group or range, or all visiting UEs from the HPLMN, and the desired data types or events, and possibly target NF types from which to collect the data or at which to subscribe for the events.

2.	V-NWDAF checks if the HPLMN is authorised to request the data based on VPLMN operator polices (that may depend on the HPLMN and may indicate permissible or restricted NF types, data types, or events).

3.	V-NWDAF collects the data from different NFs/DCCF and aggregates the collected data.

4.	V-NWDAF anonymizes or restricts the data based on VPLMN operator polices (that may depend on the HPLMN).

5.	V-NWDAF sends the processed data to H-NWDAF

6.	V-NWDAF may store the sent data for subsequent auditing or future use.

To achieve the above-defined behaviour, a new Data retrieval service is provided by the NWDAF that enables both one-time enquires (Nnwdaf_DataRetrieval_GET) based on the request/response model and event subscriptions (Nnwdaf_DataRetrieval_Subscribe) based on the Subscribe/Notify model.

6.38.2.2	VPLMN consuming analytics generated by HPLMN

Figure 6.38.2.2-1: VPLMN consuming analytics profile generated by HPLMN

1.	UDM stores the VPLMN specific user consent, operator or regulatory filters.

2.	H-NWDAF generates UE1 analytics or predictions and based on them creates or updates the "UE analytics profile" for UE1 and stores it in the H-ADRF. The H-NWDAF may send an update whenever it has new data or analytics that impacts the UE analytics profile.

3-5.	When UE1 attaches to the VPLMN, the V-NWDAF contacts H-NWDAF to retrieve the "UE analytics profile" for UE1. The V-NWDAF may retrieve the profile when the V-NWDAF determines to generate analytics for UE1 (e.g. because of a request from a VPLMN analytics consumer). The NWDAF in the VPLMN may also subscribe to notifications about updates of the UE analytics profile at the H-NWDAF. The NWDAF offers a special roaming analytics exposure service for this purpose. The H-NWDAF checks if the VPLMN is authorized to request the analytics profile based on HPLMN operator polices (that may depend on the VPLMN).

6.	H-NWDAF retrieves "UE analytics profile" of UE1 from H-ADRF.

7.	H-NWDAF retrieves VPLMN specific user consent, operator or regulatory filters from UDM and applies them to the retrieved "UE analytics profile" of UE1 to generate a "VPLMN UE analytics profile" with information on the UE1 that can be exposed to the VPLMN.

8.	H-NWDAF provides the "VPLMN UE analytics profile" of UE1 to the V-NWDAF.

9.	V-NWDAF uses the obtained "VPLMN UE analytics profile" of UE1 to extract information related to the UE1. V-NWDAF uses this information, as well as data obtained in the VLPMN related to the UE1, to generate analytics related to the UE1 and provide analytics reports to its consumers.

10.	(optional) V-NWDAF may store the "VPLMN UE analytics profile" of UE1 in the V-ADRF for future use, e.g., when UE is again served by the VPLMN. The "VLPMN UE analytics profile" stored in the V-ADRF may be an updated version of the profile obtained from H-ADRF, with updates based on the data the V-NWDAF VPLMN has collected related to the UE1, while UE1 is residing in the VPLMN.

6.38.3	Impacts on Existing Nodes and Functionality

V-NWDAF:

-	Offer new data retrieval services that enable data retrieval by NFs from another PLMN:

-	Nnwdaf_DataRetrieval_GET enabling one-time enquires based on the request/response model.

-	Nnwdaf_DataRetrieval_Subscribe enabling based on the Subscribe/Notify model,

-	Retrieve analytics profile for UE from H-NWDAF and extract and provide analytics based on it.

H-NWDAF

-	Offer new analytics retrieval services that enable analytics profiule retrieval by NFs from another PLMN:

-	Nnwdaf_AnalyticsRetrieval_GET enabling one-time enquires based on the request/response model.

-	Nnwdaf_AnalyticsRetrieval_Subscribe enabling based on the Subscribe/Notify model,

-	Generate an "UE analytics profile" for each UE subscribed at HPLMN and store it in ADRF.

-	Provide "UE analytics profiles" to the V-NWDAF after having filtered the UE analytics profile based on operator or regulatory policies and the "VPLMN user consent" restrictions for the UE.

-	Retrieve data about roaming UEs from V-NWDAF.

NRF:

-	Restrict access to other event exposure services and analytics services from outside the own PLMN and redirect to NWDAF data retrieval service at analytics retrieval service

ADRF

-	Store "UE analytics profiles" and provide them to the V-NWDAF after having filtered the UE analytics profile based on operator or regulatory policies and the "VPLMN user consent" restrictions for the UE.

6.39	Solution #39: Architecture enhancements to support Data and analytics exchange in roaming case

6.39.1	Description

This is a solution for the Key Issue#3: Data and analytics exchange in roaming case, which focuses on possible architecture enhancements.

NOTE:	The solution only addresses how home PLMN retrieve data analytics or data for roaming users in the VPLMN.

6.39.1.1	General

The proposed architecture enhancements solution is composed of the following two aspects:

-	NWDAF in Home PLMN collects data from Visited PLMN to derives analytics for roaming UE(s);

-	NWDAF consumer in Home PLMN obtains analytics from the NWDAF in Visited PLMN for roaming UE(s).

The consumer (NWDAF or NWDAF consumer) in HPLMN requests data or analytics from Visited PLMN via interworking DCCF in home PLMN and visited PLMN, respectively.

Figure 6.39.1.1-1: Roaming architectures enhancement to support Data and analytics exchange in roaming (local breakout scenario)

Figure 6.39.1.1-2: Roaming architectures enhancement to support Data and analytics exchange in roaming (home routed scenario)

6.39.2	Procedure

6.39.2.1	Procedure for NWDAF in HPLMN to collect data from VPLMN

Figure 6.39.2.1-1: Procedure for NWDAF in HPLMN to collect data from VPLMN

1.	The NWDAF service consumer subscribes to analytics information by invoking the Nnwdaf_AnalyticsSubscription_Subscribe service operation. The parameters that can be provided by the NWDAF service consumer are listed in clause 6.1.3, TS 23.288 [5].

2.	If the Target of Analytics Reporting is one UE, or a group of UE or any UE, that is roaming from the HPLMN to the visited PLMN, and the NWDAF in HPLMN needs the data related to the roaming UE(s) in the visited PLMN to derive the analytics for the analytics ID, then the NWDAF requests data collection to the DCCF in VPLMN by invoking Ndccf_DataManagement_Subscribe, including roaming indication, VPLMN ID, target area in VPLMN, event ID(s), Target of Event Reporting (UE ID, UE list, or any UE).

Editor's note:	How to determine the VPLMN ID by HPLMN NWDAF is FFS.

3.	(Optional)According to data collection request from the HPLMN NWDAF, the DCCF in HPLMN discovers the DCCF in visited PLMN from NRF based on VPLMN ID.

4.	HPLMN DCCF requests data collection to the VPLMN DCCF by invoking Ndccf_DataManagement_Subscribe, including roaming indication, HPLMN ID, VPLMN ID, target area in VPLMN, event ID(s), Target of Event Reporting (UE ID, UE list, or any UE).

5.	VPLMN DCCF determines if the requested data is available at itself. If the data is available, then the VPLMN DCCF performs step 10 to sends the data to the HPLMN by invoking Ndccf_DataManagement_Notify. If not, then the VPLMN DCCF performs step 6.

Before the step 5, the VPLMN DCCF may need to check if the consumer from HPLMN is authorized to obtain data from the VPLMN, taking the VPLMN ID, HPLMN ID, target area in VPLMN, Event ID(s) into account based on the roaming agreements, VPLMN policies and regulatory constraints between hPLMN and vPLMN and etc.

Editor's note:	It is FFS whether a new service may be appropriate to trigger the special checking by the VPLMN DCCF.

Editor's note:	It is FFS how the DCCF obtains applicable policies for the peer PLMN.

6.	(Optional) Based on the target AOI in the VPLMN and the requested event ID(s), the VPLMN DCCF discovers the target data provider(s) in the VPLMN via NRF.

7.	The VPLMN DCCF subscribes to the data provider(s) the requested data by invoking, e.g. Nnf_EventExposure_Subscibe, indicating the Target of Event Reporting is a certain UE or a group of UE or any UE roaming from the HPLMN.

8.	The data providers collect data for the roaming UE(s) from the HPLMN.

9.	The data providers send the collected data to the VPLMN DCCF by invoking, e.g. Nnf_EventExposure_Notify.

10.	The VPLMN DCCF send the collected data to the HPLMN DCCF by invoking Ndccf_DataManagement_Notify.

11.	The HPLMN DCCF send the collected data to the HPLMN NWDAF by invoking Ndccf_DataManagement_Notify.

12.	Based on the collected data from the VPLMN for the roaming UE(s), the HPLMN NWDAF derives the requested analytics for the roaming UE(s).

13.	The HPLMN NWDAF sends the generated analytics to the NWDAF consumer.

6.39.2.2	Procedure for NWDAF consumer in HPLMN to obtain analytics from VPLMN NWDAF

Figure 6.39.2.2-1: Procedure for NWDAF consumer in HPLMN to obtain analytics from VPLMN NWDAF

1.	If the Target of Analytics Reporting is one UE, or a group of UE or any UE, that is roaming from the HPLMN to the visited PLMN, the HPLMN NWDAF service consumer subscribes to analytics information by invoking the Ndccf_DataManagement_Subscribe service operation to the HPLMN DCCF, including roaming indication, VPLMN ID, target area in VPLMN, analytics ID(s), Target of Analytics Reporting (UE ID, UE list, or any UE).

2.	(Optional)According to analytics request from the HPLMN NWDAF consumer, the DCCF in HPLMN discovers the DCCF in visited PLMN from the NRF based on the VPLMN ID.

3.	HPLMN DCCF requests the analytics to the VPLMN DCCF by invoking Ndccf_DataManagement_Subscribe including roaming indication, VPLMN ID, target area in VPLMN, analytics ID(s), Target of Analytics Reporting (UE ID, UE list, or any UE).

4.	VPLMN DCCF determines if the requested analytics is available at itself. If the analytics is available, then the VPLMN DCCF performs step 9 to sends the analytics to the HPLMN by invoking Ndccf_DataManagement_Notify. If not, then the VPLMN DCCF performs step 5.

Before the step 4, the VPLMN DCCF may need to check if the consumer from HPLMN is authorized to obtain analytics from the VPLMN, taking the VPLMN ID, HPLMN ID, target area in VPLMN, analytics ID(s) into account based on the roaming agreements, VPLMN policies and regulatory constraints between hPLMN and vPLMN and etc.

5.	(Optional) Based on the target AOI in the VPLMN and the requested analytics ID(s), the VPLMN DCCF discovers the target NWDAF in the VPLMN via NRF.

6.	The VPLMN DCCF subscribes to the VPLMN NWDAF the requested analytics by invoking, Nnwdaf_AnalyticsSubscription_Subscribe, including roaming indication, VPLMN ID, target area in VPLMN, analytics ID(s), Target of Analytics Reporting (UE ID, UE list, or any UE).

7.	The VPLMN NWDAF collects data and derives the requested analytics for the roaming UE(s) from the HPLMN.

8.	The VPLMN NWDAF sends the analytics result to the VPLMN DCCF by invoking, e.g. Nnwdaf_AnalyticsSubscription_Notify.

9.	The VPLMN DCCF send the analytics result to the HPLMN DCCF by invoking Ndccf_DataManagement_Notify.

10.	The HPLMN DCCF send the analytics result to the HPLMN NWDAF consumer by invoking Ndccf_DataManagement_Notify.

6.39.3	Impacts on existing nodes and functionality

Consumer in HPLMN (e.g. HPLMN NWDAF or HPLMN NF consumer):

-	For UE(s) roaming from the HPLMN to the visited PLMN, subscribes data/analytics information to the HPLMN DCCF, indicating roaming indication, VPLMN ID, target area in VPLMN, analytics ID(s), Target of Analytics Reporting (UE ID, UE list, or any UE).

H-DCCF:

-	discovers the DCCF in visited PLMN from NRF based on VPLMN ID;

- -	requests data collection to the VPLMN DCCF, indicating roaming indication, HPLMN ID, VPLMN ID, target area in VPLMN, event ID(s), Target of Event Reporting (UE ID, UE list, or any UE)

V-DCCF:

-	checks if the consumer from HPLMN is authorized to obtain data/analytics from the VPLMN taking the VPLMN ID, HPLMN ID, target area in VPLMN, Event ID(s) into account

-	collects data/ analytics from data provider(s) for the roaming UE(s) from the HPLMN

-	sends the collected data/analytics to the VPLMN DCCF.

6.40	Solution #40: Data and analytics exchange for roaming UEs

6.40.1	Description

In roaming scenario, the HPLMN/VPLMN may cooperate to collect data or to produce analytics for a particular UE. The level of detailed data to be collected from the other PLMN or analytics to be exposed by the other PLMN can be limited based on the user consent, operator policy, regulatory constraints and/or roaming agreements. For example, an NWDAF in the HPLMN may request input data for observed service experience of a UE to NFs or NWDAF in the VPLMN to produce more accurate analytics for a home-routed traffic.

In the solution, the NWDAF in HPLMN/VPLMN accesses the NFs or NWDAF in the other PLMN via NEF for data collection or analytics subscription and behaves as an untrusted AF for the other PLMN. It is assumed in the solution that the address of the NEF in the other PLMN is preconfigured at the NWDAF in HPLMN/VPLMN as part of roaming agreement or can be discovered through the NRF in HPLMN/VPLMN depending on the operator policy, regulatory constraints and/or roaming agreements.

Editor's note:	For analytics consumed in the VPLMN, it is FFS whether the analytics should be retrieved from the HPLMN NWDAF or the VPLMN NWDAF should retrieve raw data from the HPLMN

6.40.2	Procedures

The procedure for data collection from NFs and/or NWDAFs in VPLMN is depicted in 6.40.2-1.

Figure 6.40.2-1: Procedure for data collection from VPLMN

1.	A Consumer NF in HPLMN request/subscribe analytics to the NWDAF in HPLMN. The message includes Target of Analytics Reporting that is set to specific UEs, a group of UE(s) or any UE. In case that a group of UE(s) or any UE is used as Target of Analytics Reporting, the message also includes an indication for roaming UE and the PLMN ID of VPLMN.

2.	The NWDAF in HPLMN decides to collect data and/or analytics from the NF(s) and/or NWDAF(s) in VPLMN, based on the information received from the Consumer NF.

3-4.	The NWDAF in HPLMN subscribes to NF(s) in VPLMN via NEF in VPLMN for data collection. The NEF in VPLMN verifies user consent, operator policy, regulatory constraints and/or roaming agreements, and, if the NWDAF in HPLMN is authorized to request the data form VPLMN, it forwards the subscription request to the relevant NF(s). The NEF can refer to the NSSF or AMF if the subscription request from the NWDAF in HPLMN includes HPLMN S-NSSAI(s), and translate HPLMN S-NSSAI(s) to corresponding VPLMN S-NSSAI(s) before forwarding subscription request to the NF(s).

Editor's note:	It is FFS whether NEF in VPLMN has access to user consent. It is FFS how NEF realizes that checks of operator policy, regulatory constraints and/or roaming agreements are required. It is FFS if NEF can expose the input data for all types of analytics or extensions are required.

5-6.	The NWDAF in HPLMN subscribes to NWDAF(s) in VPLMN via NEF in VPLMN for analytics. The NEF in VPLMN verifies user consent, operator policy, regulatory constraints and/or roaming agreements, and, if the NWDAF in HPLMN is authorized to request the analytics form VPLMN, it forwards the subscription request to the relevant NWDAF(s). The NEF can refer to the NSSF or AMF if the subscription request from the NWDAF in HPLMN includes HPLMN S-NSSAI(s), and translate HPLMN S-NSSAI(s) to corresponding VPLMN S-NSSAI(s) before forwarding subscription request to the NF(s).

7.	The NWDAF in HPLMN receives input data from NF(s) in VPLMN via NEF.

8.	The NWDAF in HPLMN receives analytics form the NWDAF(s) in VPLMN via NEF.

9.	The NWDAF in HPLMN generates the analytics output requested by the consumer NF.

10.	The NWDAF provides the analytics output to the consumer NF.

The procedure for data collection from NFs and/or NWDAFs in HPLMN is depicted in 6.40.2-2.

Figure 6.40.2-2: Procedure for data collection from HPLMN

1.	A Consumer NF in VPLMN request/subscribe analytics to the NWDAF in VPLMN. The message includes Target of Analytics Reporting that is set to specific UEs, a group of UE(s) or any UE. The message also includes an indication for roaming UE and the PLMN ID of HPLMN.

2.	The NWDAF in VPLMN decides to collect data and/or analytics from the NF(s) and/or NWDAF(s) in HPLMN, based on the information received from the Consumer NF. The NWDAF in VPLMN can refer to the NSSF or AMF if the subscription request from the Consumer NF includes VPLMN S-NSSAI(s), and translate VPLMN S-NSSAI(s) to corresponding HPLMN S-NSSAI(s) before sending subscription request to the NEF in HPLMN.

3-4.	The NWDAF in VPLMN subscribes to NF(s) in HPLMN via NEF in HPLMN for data collection. The NEF in HPLMN verifies user consent, operator policy, regulatory constraints and/or roaming agreements, and, if the NWDAF in VPLMN is authorized to request the data form HPLMN, it forwards the subscription request to the relevant NF(s).

Editor's note:	It is FFS how NEF realizes that checks of operator policy, regulatory constraints and/or roaming agreements are required. It is FFS if NEF can expose the input data for all types of analytics or extensions are required.

5-6.	The NWDAF in HPLMN subscribes to NWDAF(s) in VPLMN via NEF in HPLMN for analytics. The NEF in HPLMN verifies user consent, operator policy, regulatory constraints and/or roaming agreements, and, if the NWDAF in VPLMN is authorized to request the analytics form VPLMN, it forwards the subscription request to the relevant NWDAF(s).

7.	The NWDAF in VPLMN receives input data from NF(s) in HPLMN via NEF.

8.	The NWDAF in VPLMN receives analytics form the NWDAF(s) in HPLMN via NEF.

9.	The NWDAF in VPLMN generates the analytics output requested by the consumer NF.

10.	The NWDAF provides the analytics output to the consumer NF.

6.40.3	Impacts on Existing Nodes and Functionality

Consumer NF:

-	Support to include indication for roaming UE and the PLMN ID

NWDAF:

-	Support translation of S-NSSAI(s)

NWF:

-	Support translation of S-NSSAI(s)

-	Support to check user consent, operator policy, regulatory constraints and/or roaming agreements

6.41	Solution #41: Sending data that is about to be purged

6.41.1	Description

This solution is solving the problem with a data producing NF or NWDAF having heavy load or performance (e.g. due to shortage of memory) to store data or that it will soon purge data for any other reason. An NF may have light load or performance (e.g. due to reserved certain amount of memory or low on memory) for data to be stored for each subscription of data that has been muted.

6.41.2	Procedures

This solution re-use existing procedures.

Figure 6.41.2-1: Procedures of the solution

1.	The data producer (any NF storing data - subject to mute or similar) determines to soon purge data subscribed to by data consumer NF (NWDAF or DCCF).

2.	The data producer does the following.

-	In the case of a data producer NF being subscribed to with mute (Deactivate notification flag is set) it performs an Nnf_EventExposure_Notify service operation including the data collected and the cause code why sending the data.

NOTE:	In the case of NWDAF, DCCF being subscribed for Data and formatting and pre-processing instructions is instructed it performs an Nnwdaf_DataManagement_Notify service operation including fetch instruction, according as described in TS 23.288 [5].

3.	The data producer purges data.

6.41.3	Impacts on services, entities and interfaces

Data producer:

-	Adding a cause (e.g. heavy NF load or performance) in the exposure explaining why data is sent even when Deactivate notification flag is set

6.42	Solution #42: Storage and retrieval of trained ML models to/from ADRF

6.42.1	Description

This solution addresses KI#4: How to Enhance Data collection and Storage.

In addition to ADRF functionalities defined in Rel-17, the ADRF supports the following functionalities:

-	Trained ML model(s) file and ML model file serialization format stored by NF consumer i.e., NWDAF containing MTLF

-	Trained ML model(s) file retrieval by NF consumer i.e., NWDAF containing AnLF

The solution specifies how trained ML model(s) are stored by NWDAF containing MTLF and retrieved by NWDAF containing AnLF as described in clause 6.42.2.1 and clause 6.42.2.2 respectively.

Editor's note:	The aspects of trained ML model(s) storage/retrieval to/from ADRF by NWDAF should be aligned with Key Issue #5.

6.42.2	Procedures

6.42.2.1	Procedure for trained ML model(s) storage in ADRF

The procedure in Figure 6.42.2.1-1 illustrate the mechanisms to store ML model(s) in the ADRF by the NF consumer i.e., NWDAF containing MTLF.

Figure 6.42.2.1-1: Procedure for trained ML model(s) storage in ADRF

Editor's note:	How ML model privacy is supported by ADRF to consumers belonging to different vendors is in scope of SA WG3.

Editor's note:	ML model storage in ADRF triggered by NWDAF AnLF is for FFS.

If trained ML model(s) storage in ADRF is triggered by NWDAF containing AnLF then steps 1 and 2 are performed.

1.	The NWDAF containing AnLF sends Nnwdaf_MLModelInfo_Request with the following input parameters Analytics ID(s), ML model file specific information (ML model file serialization format), Notification end point address (ADRF) to the NWDAF containing MTLF.

2.

The NWDAF containing MTLF sends Nadrf_MLModelManagement_StorageRequest with input parameters Analytics ID(s), Trained ML model file(s), ML model file specific information (ML model file serialization format).

3.

The ADRF may subscribe to ML model training update with the NWDAF containing MTLF. It sends Nnwdaf_MLModelProvision_Subscribe with input parameters Analytics ID(s), ML model file specific Information (ML model file serialization format).

4.

When the ML model for which the ADRF has subscribed for ML model training update has been updated, the NWDAF containing MTLF sends Nnwdaf_MLModelProvision_Notify with following parameters Analytics ID, Trained ML model file, Notification Correlation ID.

5.

The NWDAF containing MTLF sends Nnwdaf_MLModelInfo_Response with the following parameters Analytics ID(s), Trained ML model file address, ADRF storage status which indicates if the ML model storage requested in step 1 was successful or failed

If trained ML model(s) storage in ADRF is initiated by ADRF then steps 6 and 7are performed. The trigger to initiate storage of trained ML model(s) from NWDAF MTLF is followed by steps 6 and 7.

6.	The ADRF sends Nnwdaf_MLModelProvision_Subscribe with the following input parameters ML model file specific information (ML model file serialization format).

7.	The NWDAF containing MTLF sends Nnwdaf_MLModelProvision_Notify with the following input parameters Analytics ID, Trained ML model file, Notification Correlation ID.

6.42.2.2	Procedure for trained ML model(s) retrieval from ADRF

The procedure in Figure 6.42.2.2-1 illustrate the mechanisms to retrieve ML model(s) from the ADRF by the NF consumer i.e., NWDAF containing AnLF.

Figure 6.42.2.2-1: Procedure for trained ML model(s) retrieval from ADRF

1.	NWDAF containing AnLF sends Nadrf_MLModelManagement_RetrievalRequest which includes Analytics ID(s), ML Model Filter Info (ML model file specific information), optionally Target NF (NWDAF containing MTLF) to subscribe for notifications. The ML model file specific information includes the ML model file serialization format requested by the NWDAF containing AnLF.

2.	The ADRF determines if the ML model file for the Analytics ID(s) requested is already stored. If the ML model file for the Analytics ID(s) requested in not stored in ADRF then step 3, 4, 5, 6 are performed, before these steps, the ADRF discovers the target MTLF from the NRF optionally if it isn't informed by the AnLF in the step 1. If the ML model file for the Analytics ID(s) requested in stored in ADRF the steps 3, 4, 5, 6 are skipped.

3.	ADRF sends Nnwdaf_MLModelProvision_Request with the input parameters defined in TS 23.288 [5] and additional input parameters ML model file specific information (ML model file serialization format).

4.	The NWDAF containing MTLF sends a Nnwdaf_MLModelProvision_Response with following parameters Analytics ID(s), Trained ML model file(s).

5.	ADRF sends Nnwdaf_MLModelTrainingUpdate_Subscribe with the input parameters Analytics ID(s), ML model file specific information (ML model file serialization format).

6.	When the ML model for which the ADRF has subscribed for ML model training update has been updated, the NWDAF containing MTLF sends Nnwdaf_MLModelTrainingUpdate_Notify with the following parameters Analytics ID, Trained ML model(s) file, Notification Correlation ID.

7.	The ADRF sends a response back to NWDAF containing AnLF using Nadrf_MLModelManagement_Retrieval Response with the following parameters ML Model File Information (Trained ML model(s) file, ML model file serialization format, Trained ML Model ID per Analytics ID).

8.	The NWDAF containing AnLF subscribes to ADRF using Nadrf_MLModelManagement_RetrievalTrainingUpdate_Subscribe service operation containing input parameters Trained ML Model ID per Analytics ID.

9.	The ADRF sends a notification to NWDAF containing AnLF using Nadrf_MLModelManagement_RetrievalTrainingUpdate_Notify service operation containing following parameters ML Model File Information (Trained ML model(s) file, ML model file serialization format, Trained ML Model ID per Analytics ID).

10.	NWDAF containing AnLF determines that the ML model training update is no longer required.

11.	The NWDAF containing AnLF sends Nadrf_MLModelManagement_RetrievalTrainingUpdate_Unsubscribe with Subscription Correlation ID as input parameters.

12.	ADRF determines if any of the NF consumer(s) have subscription for ML Model training update per Analytics ID. If none of the NF consumer(s) have subscription for ML model training update per Analytics ID, the ADRF removes the ML model file and ML model file specific information and proceed to step 9.

13.	ADRF sends Nnwdaf_MLModelTrainingUpdate_Unsubscribe to ADRF with the Subscription Correlation ID as input parameter.

6.43	Solution #43: ML model storage in ADRF and ML model provision from ADRF

6.43.1	Description

This solution addresses Key Issue #4 "How to Enhance Data collection and Storage" and Key Issue #5 "Enhance trained ML Model sharing".

An NWDAF containing MTLF may be consumed by multiple ML model consumers at the same time, resulting in repeated consumption signalling and data traffic of ML models with the same Analytics ID.

Thus, the NWDAF containing MTLF can store trained ML models into ADRF directly. Or, When the DCCF/MFAF is deployed in the network, the NWDAF containing MTLF can store trained ML models into ADRF via DCCF/MFAF.

Meanwhile, the NWDAF containing MTLF can register the trained ML model of the Analytics ID it supports into NRF or DCCF, the registered ML profile may include the ID of NWDAF containing MTLF, Analytics ID(s), ML model information.

The ML model consumers (e.g. NWDAF containing AnLF) can subscribe/request the needed ML model from ADRF directly or via DCCF.

6.43.2	Procedures

Figure 6.43.2-1: Procedure of ML model storage in ADRF and ML model provision from ADRF

1.	An NWDAF containing MTLF instance has the trained ML Model(s).

2.	The NWDAF containing MTLF requests to store the ML Model to the ADRF by invoking the Nadrf_MLModelManagement_StorageRequest (containing the trained ML model(s) and/or ML model(s) information) service operation or the NWDAF containing MTLF stores the trained ML Model to the ADRF via DCCF.

3.	The ADRF stores the trained ML model(s) and/or the ML model(s) information sent by the NWDAF containing MTLF. The ADRF may be based on implementation, determines whether the same trained ML Model is already stored by the NWDAF containing MTLF. If the trained ML Model is already stored, the ADRF decides to store again to update the trained ML Model sent by the NWDAF containing MTLF.

4.	The ADRF sends Nadrf_MLModelManagement_StorageRequest Response message to the NWDAF containing MTLF indicating that the trained ML Model is stored, including when the ADRF may have determined at step 3 that the trained ML Model is already stored.

5.	The NWDAF containing MTLF and/or the ADRF requests to register ML Model profile to DCCF by invoking the Ndccf_MLModelManagement_Register. Or the NWDAF containing MTLF and/or the ADRF registers its ML Model profile to the NRF by invoking the Nnrf_NFManagement_NFRegister.

The ML Model profile may include one of the following parameters: NWDAF ID, ADRF ID, Analytics ID(s), model framework, model platform,model type, model algorithm, model compilation language, model Spatial validity, model validity period, model accuracy, model space effectiveness, etc.

6.	The DCCF responds to the NWDAF containing MTLF and/or the ADRF with a Ndccf_MLModelManagement_Register Response. Or The NRF responds to the NWDAF containing MTLF and/or the ADRF with a Nnrf_NFManagement_NFRegister Response.

7.	The ML model consumer (i.e. an NWDAF containing AnLF) subscribes or requests a (set of) trained ML Model(s) associated with a (set of) Analytics ID(s) to DCCF or ADRF.

NOTE:	The procedure of NWDAF discovery and selection for trained ML models via NRF is defined in TS 23.288[5].

8.	The ADRF or DCCF notifies the ML model consumer with the trained ML Model Information (containing a (set of) file address of the trained ML model).

6.43.3	Impacts on Existing Nodes and Functionality

NWDAF containing MTLF:

-	Supports sending the trained ML model to ADRF.

-	Supports registering the ML Model profile to DCCF.

ADRF:

-	Supports Nadrf_MLModelManagement service to store the ML models.

-	Supports Nadrf_MLModelProvision or Nadrf_MLModelInfo_Request service to send the ML models.

DCCF:

-	Supporst Ndccf_MLModelManagement service to register the ML models.

  - 6.44	Solution #44: DCCF Reselection when multiple instances of DCCF exist in a network

    - 6.44.1	Description

The proposed solution is based on the following principles:

-	Multiple instances of DCCF are operating in one PLMN where each instance can be configured to collect data from any combination of Serving Area Information, or NF types/set ID of data sources.

-	The network is configured in a way that service consumers will interact with data sources through DCCF.

-	Reselection is initiated by service consumers to unsubscribe from source DCCF and subscribe to target DCCF.

-	A data source NF is bounded to a specific DCCF instance or set as defined in Rel-17.

-	Source DCCF notifies the service consumer if it cannot serve a UE anymore.

-	Service consumer unsubscribes from the data collection to source DCCF.

Each instance of DCCF can be identified using a discovery mechanism through NRF which is specified in TS 23.501 [2]. Moreover, a service consumer NF e.g., another DCCF, can use information about the Serving Area to query a DCCF.

When a service consumer NF receives notification from the source DCCF about not being able to serve the UE, it unsubscribes from the source DCCF. Then the source DCCF will unsubscribe from data sources and ensure all collected data is sent to the service consumer NF.

NOTE 1:	All serving areas are logical areas within one PLMN. Each serving area will have its own instance of DCCF and optionally other NFs e.g., data providers, etc.

NOTE 2:	At UE mobility, DCCF takes no action for subscriptions on group of UEs or Any UEs.

    - 6.44.2	Procedures

The procedure in Figure 6.44.2-1 is used when a service consumer NF determines to use a DCCF, known as target DCCF. The procedure is triggered e.g., when a UE location no longer lies within the serving area of the source DCCF, so that the source DCCF will notify the service consumer that it cannot serve the UE anymore, and includes all pending data to be notified. Other internal triggers that may result in DCCF reselection are handled within NF sets as specified in TS 23.501 [2] and TS 23.502 [3].

Figure 6.44.2-1: DCCF reselection initiated by target DCCF selected by service consumer

1.	Source DCCF notifies the service consumer that, e.g., location of UE falls outside the serving area of the DCCF, so it cannot serve the UE anymore. A cause code is also added with the notification (e.g., UE moved outside DCCF serving area, if data source is added in subscription - UE moved outside data source serving area).

2.	Service consumer for the source DCCF determines to select a new instance of DCCF and discovers and selects the target DCCF.

3.	The service consumer sends a subscription request to the target DCCF using Ndccf_DataManagement_Subscribe request.

4.	[OPTIONAL] The service consumer unsubscribes from the source DCCF since it will be served by the target DCCF from now on.

5.	[OPTIONAL] Target DCCF may subscribe to relevant data source(s), if not yet subscribed.

6.44.3	Impacts on Services, Entities, and Interfaces

DCCF:

-	Notify service consumer that it cannot serve the UE, including a cause code. If there is any pending data, it is also delivered to the service consumer NF.

6.45	Solution #45: Managing Impact of Muting on NF Producer

6.45.1	Description

TS 23.288 [5] clause 6.2.7 specifies a muting mechanism whereby a data source (e.g., AMF, SMF) stores events until the event consumer requests the data using an "activate notification" flag. Variability in the number of event notifications generated per subscription, the number of subscriptions requesting muting and the duration of muting among other factors can use up the limited capacity of data sources to buffer notifications. The only control currently provided to manage storage space for buffered event notifications is that the number of stored events may be limited based on NF configuration. When this number is reached, the NF continues to store new events and deletes the oldest events (see TS 23.288 [5] clause 6.2.7.2), which means the old events are lost from the perspective of the Event Consumer NF. This solution proposes enhancements to manage the storing of notifications by data sources and provides additional recourse to a data source that is approaching its capacity limit.

In this solution, when an Event Consumer NF (e.g., NWDAF, DCCF) requests notification muting from an Event Producer NF as described in TS 23.288 [5] clause 6.2.7.2, the Event Consumer can in addition specify an instruction that indicates to the Event Producer NF the desired behavior when an exception occurs at the Event Producer NF (e.g., full buffer). The instruction contains information about:

-	Action on buffered notifications (e.g., 'Send All', 'Discard All', 'Drop Old').

-	Action on subscription (e.g., 'close', 'continue with muting', 'continue without muting').

According to TS 23.288 [5] clause 6.2.7.2, the only option now available is "drop old" and "continue with muting", in which case the dropped notifications are lost to the Event Consumer NF.

The Event Producer NF evaluates the request from the Event Consumer NF and responds to the Event Consumer NF. For example, depending on what the Event Producer NF supports, the Event Producer NF may accept or reject (with reason code) the request.

The response from the Event Producer NF may also indicate:

-	The maximum number of notifications that the Event Producer NF will store.

-	An estimate of the duration for which notifications can be buffered (e.g., it can provide buffering for up to 2 days).

The above may be based on local policy configuration at the Event Producer NF, and/or an evaluation by the Event Producer NF of its available storage resources. It can be performed on a per subscription basis or for a group of subscriptions (all or subset of subscriptions created by an Event Consumer NF at the Event Producer NF).

6.45.2	Procedures

Figure 6.45.2-1: Modifications to Procedure for muting event notification

Figure 6.45.2-1 summarizes the changes to the procedure for muting event notification specified in TS 23.288 [5] clause 6.2.7.2-1.

0.	Local policies may be configured specifying default actions and handling of requests from Event Consumers for buffered notifications.

1.	The Event Consumer NF sends a EventExposure_Subscription_Request to the Event Producer NF, including the deactivation notification flag and optionally an exception instruction request.

2.	The Event Producer NF may apply local polices to the requested exception instructions and may evaluates the resources available for buffering of notifications.

3.	The Event Producer NF sends a subscription response to the Event Consumer NF indicating the result of the Exception Request. The Event Producer NF accepts the instructions, or rejects the instructions indicating a reason (e.g. request for "continue with muting" is not allowed). The Event Producer NF may also provide the Event Consumer NF with an estimate of the number of notifications it can buffer and the duration for which notifications can be buffered.

4.	An exception occurs (e.g. Producer NF no longer has sufficient storage space available for buffering of Notifications, or otherwise determines it is approaching a resource limitation).

5.	Event Producer NF executes exception instructions and notifies the Event Consumer NF.

6.45.3	Impacts on services, entities and interfaces

-	Event Consumer NFs (DCCF, NWDAF)

-	Support sending exception instructions.

-	Event Producer NFs (AMF, SMF, etc.)

-	Support local configuration of policy for buffered notification exception handing

-	Support reception and processing of exception instructions.

6.46	Solution #46: ADRF / NWDAF Data Storage Management

6.46.1	Description

TS 23.288 [5] clause 6.2.6 specifies that data may be stored in an ADRF/NWDAF based on a request to the DCCF or NWDAF from an NWDAF or DCCF service consumer. The request may specify "ADRF Information" indicating whether the data are to be stored in an ADRF, and optionally an ADRF ID. Alternatively, data may be stored in the ADRF/NWDAF based on local configuration on the DCCF or NWDAF. Once data or analytics is stored in the ADRF/NWDAF, it remains there permanently unless some action is taken to remove it when the data or analytics is no longer needed. Over time, this may result in the ADRF/NWDAF becoming a wasteland containing a huge volume of data or analytics that serves no purpose and is no longer needed.

This solution enhances storage management through policies that consider both operator's/local configuration and instructions from consumers that request data storage.

In this solution, consumer requests for data or analytics are enhanced so they may optionally include storage handling information that indicate how the consumer would like stored data or analytics to be maintained in the ADRF/NWDAF. The storage handling information provided by a consumer is used according to a Storage Policy provisioned (e.g., by an operator) on the DCCF, NWDAF or ADRF. Specifically:

-	A Storage Policy is provisioned on the DCCF, NWDAF or ADRF. The policy specifies operator rules for storing data. This may include whether Storage Handling Information from a consumer is allowed, default rules that apply in the absence of a consumer request containing Storage Handling Information or when the requested Storage Handling is disallowed, and allowed values or range for Storage Handing Information. For example, the Storage Policy may specify that the default duration for storing data gathered from AMFs is 4 months, Storage Handling Information requests from consumers is allowed and the allowed values for Storage Handing Information sent by a consumer is 1 month to 12 months.

-	Storage Handling Information is sent by data or analytics consumers. It specifies storage instructions for data or analytics requested by a Consumer. This information is added to existing service operations used by consumers to request data and storage (Ndccf_DataManagement_Subscribe and Nnwdaf_DataManagement_Subscribe).

The Storage Policy and Storage Handling Information may indicate:

1.	How long data/analytics are to be stored,

2.	What subset of data/analytics are to be stored,

3.	A priority indication of the data so that more important data can be retained and less important data can be removed when necessary,

4.	Specification for sending (or not sending) an alert to the consumer when data removal is imminent.

A consumer request containing Storage Handing Information is evaluated in the context of Storage Policy rules that apply for the requested data/analytics. Different Storage Policy rules may apply for different data/analytics (e.g. AMF data may be stored for a longer or shorter duration than SMF data).

One or both of a Storage Policy and Storage Handling Information may be available for particular data/analytics, for a set of data/analytics or for all data/analytics to be stored. Storage Policy rules and Storage Handling Information are considered in determining the Storage Approach to be applied for requested data. The Storage Approach consists in enforcing the storage based on the Storage Policy and/or the Storage Handling Information. A precedence may be specified as part of the Storage Policy to determine whether a Storage Policy rule supersedes a Storage Handling Request.

-	If only a Storage Policy is specified (e.g., provisioned by the operator), then a Storage Policy rule applicable to the data to be stored is applied as the Storage Approach (e.g., store all AMF data for 4 months).

-	If only Storage Handling Information is specified (by a consumer), then the request is either accepted or rejected as the Storage Approach according to configuration (e.g. by the operator) for honouring consumer requests in the absence of a policy (e.g., consumer asks to store requested AMF data for 7 months, and request is accepted or rejected).

-	If both a Storage Policy and a Storage Handling Information is specified, then the request for Storage Handling is allowed within the scope of Storage Policy rules and precedence configured by the operator. For example, if a Storage Policy rule is to store all AMF data for a maximum of 4 months but a consumer that sent a Storage Handling Request asks that the data be stored for 7 months, then data storage for 4 months or for 7 months is authorized and applied as the Storage Approach according to operator configuration of Storage Policy vs Storage Handling Request precedence.

In response to a consumer request containing Storage Handling Information, the ADRF/NWDAF/DCCF provides the result of the request indicating the Storage Approach.

To execute a Storage Approach:

-	When a DCCF is used, the DCCF may keep track of the Storage Approach and the DCCF executes tasks to update the NWDAF or ADRF storage accordingly. For example, after the 4 months storing AMF data has elapsed, the DCCF sends an Nadrf_DataManagement_Delete to remove the data that is no longer needed.

-	When a DCCF is not used, the ADRF or NWDAF keeps track of the Storage Approach and executes tasks to update the NWDAF/ADRF storage.

-	When data is to be removed from the ADRF or NWDAF, the DCCF, ADRF or NWDAF may send a notification to the Consumer that the data is about to be deleted. The consumer may acknowledge the notification or send a further request to extend the data storage period.

In addition, the NWDAF/ADRF/DCCF may provide the consumer with a "Storage Transaction Identifier so that the data can be easily fetched by the consumer. For example, if a consumer has asked to store AMF notifications for 5 months, the DCCF, ADRF or NWDAF may provide a Storage Transaction Identifier that can be used by the consumer to retrieve the stored data. This saves the effort at the DCCF, ADRF or NWDAF to filter out data when a request is received per URI.

6.46.2	Procedures

The procedure from TS 23.288 [5] clause 6.2B.3 which is used by consumers (NWDAF, DCCF) to store received notifications in the ADRF is replicated below. Changes to support storage management provided by this solution are indicted in the figure in red and are described in the steps below. Note the step numbering from the current procedure has been maintained but may be updated in the normative phase.

Figure 6.46.2-1: Historical Data and Analytics Storage (with storage management) via Notifications

0a-c.	The operator configures Storage Policy on the DCCF, NWDAF or ADRF according to operator preference.

1a-b.	The Data or Analytics Consumer may provide Storage Handling Information in its request for Data/Analytics to the NWDAF or DCCF.

2a-b.	If Storage Policy is not configured on the NWDAF or DCCF, the NWDAF or DCCF sends the Storage Handling Request to the ADRF in the Nadrf_DataManagement_StorageSubscription request.

3a-c.	The NWDAF, DCCF or ADRF determines the Storage Approach based on the received Storage Handing Information and the Storage Policy.

7a-c.	The NWDAF, DCCF or the ADRF determines that stored data/analytics can be removed from the ADRF. The determination may be made according to a Storage Approach from Step 3.

7a-1 to 7c-1	As determined by the Storage Approach, the DCCF, ADRF or NWDAF may send a notification to the Consumer that stored data/analytics are about to be deleted. The consumer may acknowledge the notification, or send a further request to extend the data/analytics storage period.

10a-10b.	The DCCF or NWDAF may send an Nadrf_DataManagement_Delete request to remove the stored data/analytics.

11.	ADRF deletes the stored data/analytics.

6.46.3	Impacts on services, entities and interfaces

NWDAF:

-	Include Storage Handling Information in service operations that request data.

-	Support Storage Policy and execution of Storage Approach.

-	Support notification to consumer of impending data/analytics deletion and response from consumer.

DCCF/MFAF:

-	Include Storage Handling Information in service operations that request data.

-	Support Storage Policy and execution of Storage Approach.

-	Support notification to consumer of impending data/analytics deletion and response from consumer.

ADRF:

-	Support Storage Policy and execution of Storage Approach.

-	Support notification to consumer of impending data/analytics deletion and response from consumer.

6.47	Solution #47: Sharing models between NWDAFs

6.47.1	Description

A pre-condition to share models between NWDAFs is that business aspects are in place and interoperability tests are done. When these are done, an Interoperable Token is known by both NWDAF containing MTLF and NWDAF containing AnLF. The NWDAF containing MTLF can register into NRF the Interoperable Token. This Interoperable Token is then used when another NWDAF as ML model consumer discovers an appropriate NWDAF containing MTLF to subscribe ML Models from.

Models are software running in the NWDAF's model execution environment. A commonly used such environment is a container-based environment. This solution builds on such environment but can easily be extended to include any environment running inside an NWDAF.

Editor's note:	The definition of an ML model is FFS.

The MLModelProvision service operation is updated to be able to handle different formats, according to model serving environments in the NWDAF containing AnLF. This is done by adding the possibility for the NWDAF containing AnLF to share what environment it is running and what services it is using for communicating externally to other NFs (such as data collection), and what output format it expects.

The model file, which is referred to in the URL the NWDAF containing MTLF includes in the response to the MLModelProvision service subscription, includes the model software and a manifest.

6.47.2	Procedures

This solution re-use existing procedures.

Figure 6.47.2-1: Existing procedures for ML Model Provisioning

Only subscription is shown above, but the same can apply for request as well.

1.	The NWDAF service consumer subscribes to ML Model (s) according to existing procedure in TS 23.288 [5].

Information that is added by the NWDAF service consumer:

-	Interoperable Token

NOTE 1:	Format of the token is up to the stage 3 discussion.

-	Representative Ratio: percentage of the related UEs whose data is used in the Analytics computation, when the Target of ML Model Reporting is a group of UE(s) or any UE.

NOTE 2:	Representative ratio is used when NWDAF trains a Model for a group of UEs or any UE, where it can only use the data of the related UEs who grant the User consent. The representative ratio can help the consumer to know if the model can represent the group of UEs or any UE, thus whether the model is useful or not.

-	A container for information to the NWDAF containing MTLF including:

-	Wanted Output format from NWDAF containing MTLF (In this solution an OCI image)

-	The environment the AnLF is running on (e.g. K8s version)

-	What APIs the model shall have and make use of. Example of such APIs are API for requesting Analytics from model.

NOTE 3:	The parameters listed for the container are only for information and will not require any 3GPP work. The content of the container is not to be specified by 3GPP.

2.	NWDAF containing MTLF notifies service consumer according to existing procedure in TS 23.288 [5].

The ML Model File includes the wanted output (in this solution an OCI image) and a manifest on how to unpack into a runtime and what APIs are available.

NOTE 4:	The content of the manifest is not to be specified by 3GPP. The manifest e.g. may contain helm chart and API description, when the wanted output is an OCI image.

No further information is needed from the NWDAF containing MTLF.

Editor's note:	How ML model privacy is supported when NWDAF containing MTLF and NWDAF containing AnLF belong to different vendors is in scope of SA WG3.

6.47.3	Impacts on services, entities and interfaces

Editor's note:	This clause captures impacts on existing 3GPP nodes and functional elements.

## 6.48	Solution #48: PCF/UE re-evaluates the URSP rules according to NWDAF's analytics

### 6.48.1	Impacts of NWDAF analytics towards URSP rules

In URSP rules, the UE maps the application traffic to a PDU sessions according to Traffic Descriptor and RSD. In a specific Traffic Descriptor, for example the IP descriptor or DNN and etc, which of the RSDs will be selected depends on the RSD precedence and UE implementation.

But now, how to generate or re-evaluate the URSP rules for UE depends on the PCF internal mechanisms. Also, even if UE receives the URSP rules from PCF, the real enforcement of URSP rules is unknown to 5GC.

The analytic from NWDAF has influence on URSP rules, as described in following:

-	The DNN selection. For Analytic ID = DN performance, the NWDAF provides DN Performance Analytics which provides analytics for user plane performance (i.e. average/maximum traffic rate, average/maximum packet delay, average packet loss rate) in the form of statistics or predictions to a service consumer. And for a specific Traffic Descriptor (for example, the Application server IP address), the PCF can determine a better DNN.

-	The slice selection. For Analytics ID = "Load level information", the NWDAF provides slice load level information to a consumer NF on a Network Slice level or a Network Slice instance level or both. For a specific Traffic Descriptor, the PCF can determine a better slice based on the slice load level analytics provided by the NWDAF.

-	The access type selection. For Analytics ID = "WLAN performance" and "Service Experience", the NWDAF provides Observed Service Experience that including the 3GPP Access performance and the WLAN performance. According to these analytics, the PCF can decide the better access technology and provide the better access type in RSD for a specific Traffic Descriptor.

-	High frequency used RSDs. For a specific Traffic Descriptor, the NWDAF can provide the statistic and prediction of high frequency used of RSDs. Also, the NWDAF can provide the high frequency used of DNN, S-NSSAI, Access type and etc.

-	Modification of Route Selection Validation Criteria in RSD. If some of the RSD has worse performance under certain UE location or time range, the Route Selection Validation Criteria should be modified.

With the help of analytic or prediction from NWDAF, the PCF can update the URSP rules to UE, for example, the access type, DNN selection or slice selection.

Also, the NWDAF can provide the URSP rules experience to PCF. For example, the PCF distributes the URSP rules (TD = FQDN = 'ABC.com', RSD 1 = SSC mode 3, DNN2) to several of UEs. But the PCF needs to check the RSD enforcement frequency and application service experience after using the RSD. The NWDAF can provide the RSD frequency and the application performance after using the RSD. This may guide the PCF to re-evaluate the URSP rules, for example, some of the low precedence URSP rules are frequently used and has high performance.

### 6.48.2	PCF updates or generates URSP rules according to NWDAF’s analytics

According to the analytics from NWDAF, the PCF updates or generates the URSP rules. The update or generation of URSP rules will take the action are defined below into account:

-	Updates the RSD precedence in Traffic Descriptor, for example, the Non-3GPP Access in RSD 2 has better performance than 3GPP Access in RSD 1, so the RSD 2 should have a higher precedence.

-	Updates the elements in RSDs, for example changes the access type, DNN selection and slice selection. The PCF can also add more elements in RSDs, or remove the existing parameters in RSDs.

-	Delivery new URSP rules to UE according to NWDAF's analytics.

-	Changes the Route Selection Validation Criteria. For example, according to the analytic, some of the RSD can't be used at certain UE location or time range, because the performance of DN and S-NSSAI are worse.

### 6.48.3	Extensions of Analytic ID = "Service Experience"

In the RSD of URSP rules, both the 3GPP access, non-3GPP access and multi-Access are available for access type of PDU session. There is no performance of Access type of PDU session in Observed Service Experience. So, it is reasonable to extend the Analytic ID = "Service Experience" to add more analytics to help to re-evaluate the URSP rules.

In Table 6.48.3-1 and Table 6.48.3-2, extended analytics are listed to extend the Analytic ID = "Service Experience".

The Observed Service Experience are extended to add an Access Type of PDU session to list the service experience in different access type of PDU session when UE's PDU session is 3GPP access, non-3GPP accessor multi-Access.

When PCF subscribes the extended Analytic ID = "Service Experience", it can receive the performance in 3GPP access, non-3GPP access type and multi-Access type, and PCF can re-evaluate the access type of PDU session in URSP rules for UE.

Table 6.48.3-1: Enhanced Service Experience statistics

| Information | Description |
| --- | --- |
| Slice instance service experiences (0..max) | List of observed service experience information for each Network Slice instance. |
| > S-NSSAI | Identifies the Network Slice |
| > NSI ID (NOTE 2) | Identifies the Network Slice instance within the Network Slice. |
| > Network Slice instance service experience | Service experience across Applications on a Network Slice instance over the Analytics target period (average, variance). |
| > SUPI list (0..SUPImax) (NOTE 3) | List of SUPI(s) for which the slice instance service experience applies. |
| > Ratio (NOTE 3) | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity (NOTE 6) | Area where the Network Slice service experience analytics applies. |
| > Validity period | Validity period for the Network Slice service experience analytics as defined in clause 6.1.3. |
| Application service experiences (0..max) | List of observed service experience information for each Application. |
| > S-NSSAI | Identifies the Network Slice used to access the Application. |
| > Application ID | Identification of the Application. |
| > Service Experience Type | Type of Service Experience analytics, e.g. on voice, video, other. |
| > UE location (NOTE 1, NOTE 5) | Indicating the UE location information (e.g. TAI list, gNB ID, etc) when the UE service is delivered. |
| > UPF Info (NOTE 4) | Indicating UPF serving the UE. |
| > DNAI | Indicating which DNAI the UE service uses/camps on. |
| > DNN (NOTE 4) | DNN for the PDU Session which contains the QoS flow. |
| > Application Server Instance Address | Identifies the Application Server Instance (IP address of the Application Server) or FQDN of Application Server. |
| > Service Experience | Service Experience over the Analytics target period (average, variance). |
| > SUPI list (0..SUPImax) (NOTE 3) | List of SUPI(s) with the same application service experience. |
| > Ratio (NOTE 3) | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity (NOTE 6) | Area where the Application service experience analytics applies. |
| > Validity period | Validity period for the Application service experience analytics as defined in clause 6.1.3. |
| > RAT Type (NOTE 7) | Indicating the list of RAT type(s) for which the application service experience analytics applies. |
| > Access Type of PDU session | Indicates the Access Type of a PDU Session for the matching Application service traffic. |
| > Frequency (NOTE 7) | Indicating the list of carrier frequency value(s) of UE's serving cell(s) where the application service experience analytics applies. |
| NOTE 1: This information element is an Analytics subset that can be used in "list of analytics subsets that are requested. NOTE 2: The NSI ID is an optional parameter. If not provided the Slice instance service experience indicates the service experience for the S-NSSAI. NOTE 3: The SUPI list and Ratio in the service experience information for an application can be omitted, if the corresponding parameter(s) is/are provided and are assigned with the same value(s) in the service experience information for the slice instance which the application belongs to. Otherwise, the SUPI list and Ratio are mandatory to be provided for an application service experience. NOTE 4: If the consumer NF is an AF, the item "DNN" and "UPF info" shall not be included, and the NEF is responsible for translation of SUPI to GPSI, internal group identifiers to external ones, and UE location to geographical area, by querying UDM, prior to contacting the AF. NOTE 5: When possible and applicable to the access type, UE location is provided according to the preferred granularity of location information. UE location shall only be included if the Consumer analytics request is for single UE or a list of UEs. Inclusion of UE location requires user consent. NOTE 6: The Spatial validity is present in the output parameters if the consumer provided the Area of Interest as defined in Table 6.4.1-1. NOTE 7: When "any" value has been provided in the request (e.g. "any" RAT type, "any" frequency, or "any" for all the RAT type and frequency indication), the NWDAF provides an instance of the Application service experience per combination of RAT Type(s) and/or Frequency value(s) having the same Service Experience. |  |

Table 6.48.3-2: Enhanced Service Experience predictions

| Information | Description |
| --- | --- |
| Slice instance service experiences (0..max) | List of observed service experience information for each Network Slice instance. |
| > S-NSSAI | Identifies the Network Slice |
| > NSI ID (NOTE 2) | Identifies the Network Slice instance within the Network Slice. |
| > Network Slice instance service experience | Service experience across Applications on a Network Slice instance over the Analytics target period (average, variance). |
| > SUPI list (0..SUPImax) (NOTE 3) | List of SUPI(s) for which the slice instance service experience applies. |
| > Ratio (NOTE 3) | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity (NOTE 6) | Area where the Network Slice service experience analytics applies. |
| > Validity period | Validity period for the Network Slice service experience analytics as defined in clause 6.1.3. |
| > Confidence | Confidence of this prediction. |
| Application service experiences (0..max) | List of predicted service experience information for each Application. |
| > S-NSSAI | Identifies the Network Slice used to access the Application. |
| > Application ID | Identification of the Application. |
| > Service Experience Type | Type of Service Experience analytics, e.g. on voice, video, other. |
| > UE location (NOTE 1, NOTE 5) | Indicating the UE location information (e.g. TAI list, gNB ID, etc) when the UE service is delivered. |
| > UPF Info (NOTE 4) | Indicating UPF serving the UE. |
| > DNAI | Indicating which DNAI the UE service uses/camps on. |
| > DNN (NOTE 4) | DNN for the PDU Session which contains the QoS flow. |
| > Application Server Instance Address | Identifies the Application Server Instance (IP address of the Application Server) or FQDN of Application Server. |
| > Service Experience | Service Experience over the Analytics target period (average, variance). |
| > SUPI list (0..SUPImax) (NOTE 3) | List of SUPI(s) with the same application service experience. |
| > Ratio (NOTE 3) | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity (NOTE 6) | Area where the Application service experience analytics applies. |
| > Validity period | Validity period for the Application service experience analytics as defined in clause 6.1.3. |
| > Confidence | Confidence of this prediction. |
| > RAT Type (NOTE 7) | Indicating the list of RAT type(s) for which the application service experience analytics applies. |
| > Access Type of PDU session | Indicates the Access Type of a PDU Session for the matching Application service traffic. |
| > Frequency (NOTE 7) | Indicating the list of carrier frequency value(s) of UE's serving cell(s) where the application service experience analytics applies. |
| NOTE 1: This information element is an Analytics subset that can be used in "list of analytics subsets that are requested". NOTE 2: The NSI ID is an optional parameter. If not provided the Slice instance service experience indicates the service experience for the S-NSSAI. NOTE 3: The SUPI list and Ratio in the service experience information for an application can be omitted, if the corresponding parameter(s) is/are provided and are assigned with the same value(s) in the service experience information for the slice instance which the application belongs to. Otherwise, the SUPI list and Ratio are mandatory to be provided for an application service experience. NOTE 4: If the consumer NF is an AF, the item "DNN" and "UPF info" shall not be included, and the NEF is responsible for translation of SUPI to GPSI, internal group identifiers to external ones, and UE location to geographical area, by querying UDM, prior to contacting the AF. NOTE 5: When possible and applicable to the access type, UE location is provided according to the preferred granularity of location information. UE location shall only be included if the Consumer analytics request is for single UE or a list of UEs. Inclusion of UE location requires user consent. NOTE 6: The Spatial validity is present in the output parameters if the consumer provided the Area of Interest as defined in Table 6.4.1-1. NOTE 7: When "any" value has been provided in the request (e.g. "any" RAT type, "any" frequency, or "any" for all the RAT type and frequency indication), the NWDAF provides an instance of the Application service experience per combination of RAT Type(s) and/or Frequency value(s) having the same Service Experience. |  |

The NWDAF provides the statistic or prediction of service experience information for each Application under different access type of PDU session, for example, the 3GPP Access, non-3GPP Access or multi-Access.

### 6.48.4	Newly introduce Analytic ID = “URSP rules experience”

This clause specifies how NWDAF can provide URSP rules experience (i.e. in a certain Traffic descriptor the frequency of the RSD enforcement, the application performance after using the RSD, or the performance of each route selection component related) analytics, in the form of statistics or predictions, to a service consumer (for example, the PCF).

Editor's note:	Whether and how to collect the URSP rule enforcement is FFS.

The URSP rules experience analytics may provide one or more of the following outputs:

-	RSD enforcement frequency in a certain Traffic Descriptor, URSP rules;

-	Application service experience when using certain RSDs in the Traffic Descriptor;

-	Performance of the route selection component in certain Traffic Descriptor, URSP rules;

The consumer of these analytics shall indicate in the request or subscription:

-	Analytics ID = "URSP rules experience";

-	Target of Analytics Reporting: one or more SUPI(s) or Internal Group Identifier(s), or "any UE";

-	Analytics Filter Information;

-	optionally, maximum number of objects and maximum number of SUPIs;

Table 6.48.4-1: Input related to the URSP rules experience

| Information | Source | Description |
| --- | --- | --- |
| Application descriptors | PCF | It consists of OSId and OSAppId(s). |
| IP descriptors | PCF | Destination IP 3 tuple(s) (IP address or IPv6 network prefix, port number, protocol ID of the protocol above IP). |
| Domain descriptors | PCF | FQDN(s) or a regular expression which are used as a domain name matching criteria. |
| Non-IP descriptors | PCF | Descriptor(s) for destination information of non-IP traffic |
| DNN | PCF | This is matched against the DNN information provided by the application. |
| Connection Capabilities | PCF | This is matched against the information provided by a UE application when it requests a network connection with certain capabilities. |
| NOTE: The definition of information are referred to Table 6.6.2.1-2 in TS 23.503 [4]. |  |  |

The output of Analytic ID = "URSP rules experience" are listed below:

Table 6.48.4-2: URSP rules enforcement statistics

| Information | Description |
| --- | --- |
| Traffic descriptor (1…n) | List of the Traffic Descriptor in UE as defined in clause 6.6.2 of TS 23.503 [4] |
| > Application descriptors | It consists of OSId and OSAppId(s). |
| > IP descriptors | Destination IP 3 tuple(s) (IP address or IPv6 network prefix, port number, protocol ID of the protocol above IP). |
| > Domain descriptors | FQDN(s) or a regular expression which are used as a domain name matching criteria (NOTE 6). |
| > Non-IP descriptors | Descriptor(s) for destination information of non-IP traffic |
| > DNN | This is matched against the DNN information provided by the application. |
| > Connection Capabilities | This is matched against the information provided by a UE application when it requests a network connection with certain capabilities. |
| >> Route Selection Descriptor (1…n) | List of the RSDs contained in each Traffic Descriptor, that are used by single UE or several of UEs |
| >>> DNN | Indicates the Data network name |
| >>> Access type | Access Type (3GPP or non-3GPP or Multi-Access) when the UE establishes a PDU Session for the matching application. |
| >>> Network slice | A single value of S-NSSAI(s) to indicate the slice. |
| >> usage frequency | Indicating the RSD or RSD parameters (e.g.: DNN, S-NSSAI and etc) enforcement and usage frequency. |
| >> Application service experience | Indicating the application service experience when UE using certain RSD over the Analytics target period. |

Table 6.48.4-3: URSP rules enforcement Prediction

| Information | Description |
| --- | --- |
| Traffic descriptor (1…n) | List of the Traffic Descriptor in UE as defined in clause 6.6.2 of TS 23.503 [4] |
| > Application descriptors | It consists of OSId and OSAppId(s). |
| > IP descriptors | Destination IP 3 tuple(s) (IP address or IPv6 network prefix, port number, protocol ID of the protocol above IP). |
| > Domain descriptors | FQDN(s) or a regular expression which are used as a domain name matching criteria (NOTE 6). |
| > Non-IP descriptors | Descriptor(s) for destination information of non-IP traffic |
| > DNN | This is matched against the DNN information provided by the application. |
| > Connection Capabilities | This is matched against the information provided by a UE application when it requests a network connection with certain capabilities. |
| > Confidence | Confidence of this prediction. |
| >> Route Selection Descriptor (1…n) | List of the RSDs contained in each Traffic Descriptor, that are used by single UE or several of UEs |
| >>> DNN | Indicates the Data network name |
| >>> Access type | Access Type (3GPP or non-3GPP or Multi-Access) when the UE establishes a PDU Session for the matching application. |
| >>> Network slice | A single value of S-NSSAI(s) to indicate the slice. |
| >> usage frequency | Indicating the RSD or RSD parameters (e.g.: DNN, S-NSSAI and etc) enforcement and usage frequency. |
| >> Application service experience | Indicating the application service experience when UE using certain RSD over the Analytics target period. |

Editor's note:	It is FFS whether existing UE communication analytics and slice load analytics are sufficient to determine the use of URSP rules.

### 6.48.5	Procedures for URSP rules re-evaluation in PCF and UE

Figure 6.48.5-1: URSP rule re-evaluation for PCF according to NWDAF's analytics

1.	The PCF sends an Analytics request/subscribe (Analytics ID, Target of Analytics Reporting = single UE, group of UEs or any UE, Analytics Reporting Information=Analytics target period) to NWDAF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe. The analytics can be requested with the filter information.

The PCF subscribes the following Analytics ID for URSP rules re-evaluating:

-	Subscribes Analytics ID = "Service Experience" to receive the both 3GPP access and non-3GPP access performance. Compare the performance in each access type and determine the access type in URSP rules. Also, by setting analytics filter as Traffic Descriptor (for example, the DNN, FQDN, IP descriptor and etc), the PCF subscribes the Analytics ID = "Service Experience" to receive the statistic or prediction of RSD under this TD, for example, the frequency of used RSD, or RSD performance.

-	Subscribes Analytic ID = DN performance to receive the DN Performance Analytics to decide the DNN selection in URSP rules.

-	Subscribes Analytics ID = "Load level information" to receive the slice load level information to determine the network slice selection in URSP rules.

-	Subscribes Analytics ID = "WLAN performance" to receive the WLAN performance to determine the access type in URSP rules.

-	Subscribes Analytics ID = "URSP rules experience" to receive RSD enforcement frequency in a certain Traffic Descriptor, URSP rules; Or to receive the application service experience when using certain RSDs in the Traffic Descriptor; Or to receive the performance of the route selection component in certain Traffic Descriptor, URSP rules;

2.	(optional) The NWDAF collects the URSP rules enforcement.

Editor's note:	Whether and how to collect URSP rules enforcement is FFS and depends on the conclusion from FS_eUEPO.

3.	The NWDAF derives the analytic.

4.	The NWDAF provides the data analytics, i.e. the observed Service Experience analytics to the consumer NF (including PCF and UE) by means of either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1, indicating the results below:

-	Slice performance analytics: the NWDAF provides slice load level information to a consumer NF on a Network Slice level or a Network Slice instance level or both, see clause 6.3.3A of TS 23.288 [5] for detailed outputs. For a specific Traffic Descriptor, the NWDAF can provide a better slice which has lower slice load in RSD of this Traffic Descriptor.

-	DN performance analytics: the NWDAF provides DN Performance Analytics which provides analytics for user plane performance (i.e. average/maximum traffic rate, average/maximum packet delay, average packet loss rate) in the form of statistics or predictions to a service consumer, see clause 6.14.3 of TS 23.288 [5] for detailed outputs. And for a specific Application server IP address (can be the Traffic Descriptor), the PCF can decide a better DNN with better performance.

-	WLAN performance analytics: the NWDAF provides WLAN performance analytics to PCF, including the RSSI, RTT, UL/DL data rate of specific SSID. The PCF can decide the access type by comparing the performance between RAT access and WLAN.

-	Enhanced Observed Service Experience analytics: the NWDAF provides enhanced Observed Service Experience that including the 3GPP Access performance,the non-3GPP performance and the multi-Access performance, see clause 6.48.3 of this paper for detailed outputs. According to these analytics, the PCF can decide the better access type (3GPP access or non-3GPP access) in RSD of PDU session for a specific Traffic Descriptor.

-	URSP rules experience: For a specific Traffic descriptor, the NWDAF also provides the statistic or prediction of RSD or RSD parameters. For a certain Traffic descriptor, the NWDAF provides the URSP rules enforcement frequency and the application service experience corresponding to the RSD. The PCF can decide to generate or update the URSP rules.

5.	The PCF generates or updates the URSP rules for single UE, a group of UEs or several UEs. According to the performance and URSP rules enforcement information provided by NWDAF, the action of URSP rules re-evaluation are listed below:

-	Updates the RSD precedence in Traffic Descriptor, for example, the Non-3GPP Access in RSD 2 has better performance than 3GPP Access in RSD 1, so the RSD 2 should have a higher precedence.

-	Updates the elements in RSDs, for example changes the access type, DNN selection and slice selection. The PCF can also add more elements in RSDs, or remove the existing parameters in RSDs.

-	Delivery new URSP rules to UE according to NWDAF's analytics.

-	Changes the Route Selection Validation Criteria. For example, according to the analytic, some of the RSD can't be used at certain UE location or time range, because the performance of DN and S-NSSAI are worse.

### 6.48.6	Impacts on services, entities and interfaces

NWDAF:

-	Update Extensions of Analytic ID = "Service Experience" by adding access type of PDU session.

-	Introduce new Analytic ID that to provide URSP rules enforcement statistic or prediction under a specific Traffic Descriptor in Analytic ID = "URSP rules experience"

-	Data collection of URSP rules enforcement.

PCF:

-	PCF re-evaluates URSP rules according to NWDAF's analytics.

6.49	Solution #49: NWDAF assisted URSP decision for redundant transmission

6.49.1	Description

The solution addresses the key issue #6 "NWDAF-assisted URSP".

Apart from redundant transmission at transport layer, two redundant mechanisms are defined to provide redundant transmission for URLLC services since Rel-16: a) Dual Connectivity based end to end Redundant User Plane Paths; b) Redundant Transmission on N3/N9 interface. URSP can be applied to support the end to end Redundant transmission, i.e. the network can provide URSP rules for applications requiring redundant transmission to the UE, with different combination of DNN and S-NSSAI. The URSP is further enhanced to include RSN and PDU Session Pair ID for the redundant transmission in Rel-17. While the Redundant Transmission on N3/N9 can be initiated by the SMF based the authorized 5QI, gNB capability and local policies.

As defined in the TS 23.288 [5], Redundant Transmission Experience related analytics can assist the SMF for decision on Redundant Transmission on N3/N9 interface, by providing the UL/DL packet delay, packet drop rate to the SMF. However, how the end to end Redundant transmission can benefit from the Redundant Transmission Experience related analytics is undefined yet.

To achieve this goal, this paper proposes that the NWDAF exposes the Redundant Transmission Experience analytics to the PCF for the UE, then the PCF for the UE can determine whether to provision the corresponding URSP rules for redundant transmission to the UE. Thus, the UE can initiate redundant transmission for URLLC services. Based on this solution, the redundant E2E user plane can be triggered as needed, subjected to the network quality.

The Redundant Transmission Experience analytics as defined in TS 23.288 [5] is to be enhanced:

-	PDU Session ID is involved as new Analytics Filtering Information.

-	The NWDAF is required to collect more input data from UPF or OAM for Redundant Transmission Experience Analytics, as described in the following table, in addition to what's specified in the Table 6.13.2-1 of the TS 23.288 [5]:

Table 6.49.1-1: Packet drop and/or packet delay measurement per QFI or GTP level

| Information | Source | Description |
| --- | --- | --- |
| E2E UL/DL packet delay | UPF | End-to-End (between UE and UPF) packet delay measurements. |
| E2E UL/DL packet drop rate | OAM | End-to-End (between UE and UPF) packet drop rate measurements. |

The Observed Redundant Transmission Experience as defined in the Table 6.13.3-1 and Table 6.13.3-2 of the TS 23.288 [5] needs to be enhanced respectively, as below:

Table 6.49.1-2: Redundant Transmission Experience statistics

| Information | Description |
| --- | --- |
| > Observed Redundant Transmission Experience | Observed Redundant Transmission Experience related information during the Analytics target period. |
| >> E2E UL/DL packet drop rate | Observed End-to-End (between UE and UPF) UL/DL packet drop rate (average, variance). |
| >> E2E UL/DL packet delay | Observed End-to-End (between UE and UPF) UL/DL packet delay (average, variance). |

Table 6.49.1-3: Redundant Transmission Experience predictions

| Information | Description |
| --- | --- |
| > Predicted Redundant Transmission Experience | Predicted Redundant Transmission Experience related information during the Analytics target period. |
| >> E2E UL/DL packet drop rate | Predicted End-to-End (between UE and UPF) UL/DL packet drop rate (average, variance). |
| >> E2E UL/DL packet delay | Predicted End-to-End (between UE and UPF) UL/DL packet delay (average, variance). |

6.49.2	Procedures

NOTE 1:	The procedure is just for information to show how the PCF requests the NWDAF to provide analytics to help PCF make decision on URSP regarding E2E redundant transmission.

Figure 6.49.2-1 shows the procedure how the NWDAF can be triggered to provide the Redundant Transmission analytics to the PCF for the UE and how the Redundant Transmission analytics can be utilized by the PCF to trigger the establishment of Redundant end to end user plane paths.

Figure 6.49.2-1: Procedure for triggering end to end user plane path Redundant Transmission

1.	The PCF for the UE receives dynamic redundant transmission request from the AF, either via the Service specific information provisioning procedure as defined in step 0 to step 5 in figure 4.15.6.7-1 of 23.502 [3]. The request message includes the target AF-Service-Identifier, target UE(s) identified by the user identity or External Group ID or any UE, dynamic redundant transmission request indication, and optionally SLA requirements for the application service.

2.	The PCF for the UE sends a request to the NWDAF for analytics on the target UE, using either the Nnwdaf_AnalyticsInfo or Nnwdaf_AnalyticsSubscription_Subscribe service. The type of analytics is set to Redundant Transmission Experience. Analytics Filter Information optionally contains DNN, S-NSSAI, PDU Session ID, Area of Interest, etc. The DNN, S-NSSAI and PDU Session ID can be determined by the PCF based on local configuration or URSP rules.

3.	If the request is authorized, the NWDAF, in order to provide the requested analytics, may collect the service data information from OAM, SMF/UPF, following the procedure captured in clause 6.2.3.2 of TS 23.288 [5].

This step may be skipped when e.g. the NWDAF already has the requested analytics available.

4.	The NWDAF derives requested analytics.

5.	The NWDAF provides requested Redundant Transmission Experience analytics to the PCF for the UE, using either the Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1.

6.	Based on the received analytics result, the PCF for the UE may decide to update the URSP rules of the UE to activate E2E Redundant Transmission.

7.	The PCF for the UE provides the updated URSP rules to the UE via UE Configuration Update procedure as specified in clause 4.2.4.3 of TS 23.502 [3].

8.	Optionally, the PCF for the UE notifies the AF that the E2E Redundant Transmission can be initiated by sending E2E Redundant Transmission Availability indication via Npcf_EventExposure_Notify service operation.

NOTE 2:	The AF can further instruct the UE to trigger redundant transmission for the application, via user plane message or Application Triggering procedure as defined in clause 4.13.2.2 of TS 23.502[3].

9.	The application on the UE initiates redundant transmission based on UE's re-evaluation of the updated URSP rules.

10.	The UE initiates Redundant PDU Session Establishment/Modification procedure to establish E2E Redundant user plane paths for URLLC services.

In case the PDU Session parameters (e.g. DNN, S-NSSAI, SSC mode) of the existing PDU Session for URLLC service are not updated and the corresponding URSP rule contains RSN and/or PDU Session Pair ID, the UE initiates PDU Session Modification procedure by indicating the RSN/PDU Session Pair ID to the SMF, and initiates another Redundant PDU Session Establishment procedure by indicating the corresponding RSN/PDU Session Pair ID to the SMF.

In case the PDU Session parameters of the existing PDU Session for URLLC service are updated, the UE initiates the PDU Session Establishment procedure for both Redundant user plane, and may provide the corresponding RSN/PDU Session Pair ID, if available in the URSP, to the SMF.

6.49.3	Impacts on Existing Nodes and Functionality

NWDAF:

-	Support the enhanced analytics on Redundant Transmission Experience Analytics.

AF:

-	Support to request for dynamic Redundant Transmission user plane path.

PCF:

-	Support to subscribe or request Redundant Transmission Experience Analytics from the NWDAF.

-	Support to update URSP rules based on the Redundant Transmission Experience Analytics.

UE:

-	Support Redundant PDU Session Establishment/Modification based on the re-evaluation of URSP rules.

6.50	Solution #50: Enhancements on QoS Sustainability analytics

6.50.1	Description

This solution is proposed to address Key Issue #7: Enhancements on QoS Sustainability analytics.

The consumer of QoS Sustainability analytics may request the NWDAF analytics information regarding the QoS change statistics for an analytics target period in the past in a certain area or the likelihood of a QoS change for an Analytics target period in the future in a certain area. While in the existing solution, the certain area can only be TA/cell, a cell level QoS prediction is not precise enough for any UE within the cell, since in specific parts of a cell the experienced QoS will be less or higher than the current cell-level QoS prediction.

Thus, besides TA/cell information, a finer granularity area (e.g. below Cell level) is also introduced for NWDAF to provide with more accuracy analytics.

The procedure for below cell anylatics is illustrated in 6.50.2.

6.50.1.1	Input data

The NWDAF collects location, bit rate and delay for the UE from the sources listed in Table 6.50.1.1-1.

Table 6.50.1.1-1: UE related Network Data from 5GC NF

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | 5GC NF | A time stamp associated with the collected information. |
| Location | GMLC/LMF | The UE location information, e.g. cell ID or finer granularity below cell. |
| UE ID | AMF | (list of) SUPI(s). |
| SMF info | AMF | SMF address for the SMF serving the UE |
| UPF info | SMF | UPF address for the UPF serving the UE. |
| QFI | SMF | QoS Flow Identifier. |
| QoS flow Bit Rate | UPF | The observed bit rate for UL direction; and The observed bit rate for DL direction. |
| QoS flow Packet Delay | UPF | The observed Packet delay for UL direction; and The observed Packet delay for the DL direction. |

6.50.2	Procedures

Figure 6.50.2-1: Procedure for "QoS Sustainability" analytics in a finer granularity

1.	The NF consumer requests or subscribes to analytics information on "QoS Sustainability" provided by the NWDAF. The parameters included in the request are defined in TS 23.288 [5] clause 6.9.1.The NF can request statistics or predictions or both.

The consumer provides the TAIs or Cell IDs, and finer granularity area (e.g. blow cell via longitude/latitude range etc.) in "Location information" when requesting QoS Sustainability analytics.

2a.	If the request is authorized, and in order to provide the requested analytics, the NWDAF decides the AMF(s) based on the TAIs/Cell IDs, and obtains the UE list in the TAs/Cells from AMF by invoking Namf_EventExposure_Subscribe service operation using event ID "Number of UEs present in a geographical area" as described in TS 23.502 [3].

2b.	The NWDAF invokes Namf_EventExposure_Subscribe service operation to get the update of the UE list using event ID "UE moving in or out of Area of Interest" as described in TS 23.502 [3].

3.	The NWDAF initiates the LCS Service Request to the GMLC/LMF to get the location of UEs from UE list provided by the AMF in step 2.

4.	The GMLC/LMF initiates the UE location service procedure and gets the location of the UEs.

5.	The GMLC/LMF provides location information for each UE in the UE list to the NWDAF.

6.	From the list of UE locations returned by the GMLC/LMF, the NWDAF identifies the UEs located in finer granularity area by comparing the UEs' locations to the finer granularity area, provided in step 1.

7.	The NWDAF invokes Namf_EventExposure_Subscribe service operation to get the serving SMF for the UE.

8.	Based on the serving SMF in step 7, the NWDAF invokes Nsmf_EventExposure_Subscribe service operation to get the UPF information for the UE.

9.	The NWDAF may collect QoS information either from the UPF directly, or subscribe to the UPF via the SMF. The QoS information may include the bandwidth, packet delay for the UE.

10.	The NWDAF verifies whether the triggering conditions are met and derives the requested analytics. The NWDAF can detect the need for notification based on comparing the requested analytics of the target 5QI against the Reporting Threshold(s) provided by the consumer in any cell over the requested Analytics target period.

11.	The NWDAF provides the response or notification on "QoS Sustainability" to the NF consumer.

6.50.3	Impacts on Existing Nodes and Functionality

NWDAF:

-	Extensions of the NWDAF function to support a finer granularity area (e.g. below Cell level) analytics of with the NF consumers.

-	Extensions of the NWDAF subscription services to support location request from the GMLC/LMF.

6.51	Solution #51: Selection, Monitoring, and Maintenance of NWDAF(s) for Federated Learning in 5GC

6.51.1	Description

This solution is proposed to address Key Issue #8: Supporting Federated Learning in 5GC. The study bullets of this Key Issues include

-	Study how to coordinate multiple NWDAFs including selection of participant NWDAF instances in the Federated Learning group, e.g., assistance information (if any) to perform the selection, and decision of role for the participant NWDAF;

-	Study whether and how to perform performance (e.g. network performance and model performance) monitoring of the NWDAF Federated Learning operation.

To address the challenges in the above bullets for supporting Federated Learning in 5GC, this solution focus on the NWDAF(s) selection in Federated Learning preparation phase, NWDAF(s) monitoring and maintenance in Federated Learning execution phase.

A lot of factors influence Client NWDAF(s) selection in Federated Learning preparation phase. For example, the capability of NWDAF(s), the interoperability and availability of Client NWDAF(s) to join in Federated Learning.

In Federated Learning execution phase, due to dynamic changes of federation network, current Client NWDAF(s) may leave or join, the dynamic joining and leaving of Client NWDAF(s) to a Federated Learning multi-round learning/training process in 5GC should be considered. In addition, methods may be applied for Server NWDAF to monitoring the status changes (e.g., changes of capabilities and availability) of Client NWDAF(s).

Editor's note:	It is FFS if the existing SBA service can be resued for the procedures proposed in this solution or new SBA services should be defined.

6.51.2	Procedures

6.51.2.1	Client NWDAF(s) Selection in Federated Learning Preparation Phase

In Federated Learning preparation phase, Server and (potential) Client NWDAFs are discovered via NRF, and Client NWDAF(s) are selected by the method for handshake pattern. The Client NWDAF(s) selection is based on the availability, capability, etc.

Figure 6.51.2.1-1: Procedure for Client NWDAF(s) selection in Federated Learning preparation phase

Figure 6.51.2.1-1 illustrates the procedures of Client NWDAF(s) selection in Federated Learning preparation phase.

The procedure for NWDAFs selection is as follows:

0.	NWDAFs register into NRF with Federated Learning capability. Server NWDAF discovers Client NWDAFs based on e.g., Federated Learning capability, Analytics ID, etc.

1.	Server NWDAF sends Federated Learning preparation request with Interoperability information to Client NWDAF(s). In the preparation request, indication of the role for the NWDAF(s), i.e., act as Client NWDAF(s), may be contained.

NOTE 1:	The Interoperability information indicates what abilities (e.g., able to run certain models) are needed for the client NWDAF to support this FL procedure, e.g., if the server NWDAF and the client NWDAF can share model and how to share model. The Interoperability information is determined among different vendors and its content is not to be specified by 3GPP.

2.	Client NWDAF(s) decides whether to join the Federated Learning process based on its availability, capability and Interoperability information.

3.

Client NWDAF(s) send the response to server NWDAF indicating if it wants to join the FL procedure.

4.	The Server NWDAF may send test tasks to the Client NWDAF(s) that want to join the FL procedure. The Client NWDAF(s) run the test tasks and send the results to the Server NWDAF.

NOTE 2:	The test tasks may be micro computation or training tasks, the requirement for completing the micro tasks is the same as or is similar to the main tasks. The test task could be a small task to let the client NWDAF collect local data and sends the local model weights back to the server; or some test to make sure that the server and client NWDAF can communicate if they use the same FL framework or library. How to retrieve and run the test tasks is out of scope of 3GPP.

5.	Server NWDAF selects the Client NWDAF(s), the result of the test tasks may be taken into account by the Server NWDAF for the selection of Client NWDAF(s).

6.51.2.2	NWDAFs Monitoring and Re-selection in Federated Learning Execution Phase

In Federated Learning execution phase, Server NWDAF monitors the status changes of Client NWDAF(s). Client NWDAF(s) may be re-selected based on the updated status, availability, and/or capability, etc., of the Client NWDAF(s) for the FL tasks.

Figure 6.51.2.2-1: Procedure for NWDAFs monitoring and re-selection in Federated Learning execution phase

Figure 6.51.2.2-1 illustrates the procedure of monitoring and re-selection of Client NWDAF(s) in Federated Learning execution phase.

The procedure for monitoring and re-selection of Client NWDAF(s) is as follows:

1.	Server NWDAF monitoring the status of Client NWDAF(s) during the Federated Learning execution process, receives the updated status of the Client NWDAF(s).

Server NWDAF may perform monitoring and obtain the updated status of Client NWDAF(s) directly and/or via NRF.

NOTE 1:	The status of client NWDAF could be the NF load, NF availability, its capability changes, e.g. it does not support FL anymore

2.	Server NWDAF checks Client NWDAF(s) status based on the received information, judges whether re-selection of Client NWDAF(s) for the next round(s) of Federated Learning is needed. The judgement is based on the updated status of the Client NWDAF(s), including the availability, capability, etc.

Editor's note:	It is FFS if step 2 can be optimized, e.g., whether for every round in FL, the server NWDAF need to check every client's status.

3.	[If re-selection is needed as judged in step 2.] Server NWDAF re-select Client NWDAF(s) as steps 1-5 in Figure 6.51.2.1-1.

The procedure for discovery of new Client NWDAF(s) in Federated Learning execution phase is given in clause 6.51.2.3.

4.	Client NWDAF(s) terminates operations for the Federated Learning if it receives termination request from the Server NWDAF.

6.51.2.3	Dynamic Discovery and Joining of New NWDAF(s) in Federated Learning Execution Phase

There are two possible cases for Server NWDAF to get the information of the new Client NWDAF(s), i.e., from the new Client NWDAF(s) directly or via NRF.

6.51.2.3.1	New Client NWDAF(s) Inform Server NWDAF Directly

Figure 6.51.2.3.1-1: Procedure for dynamic discovery of new NWDAFs in Federated Learning execution phase when the information about the Server NWDAF is known at the new Client NWDAF(s)

Client NWDAFs 1 to N are selected by Server NWDAF for participating the current round of Federated Learning. Client NWDAFs N+1 to N+X, which are the new ones, have the capability to join in the next rounds of training processes.

New Client NWDAF(s), which are available and/or have the capability to join in the Federated Learning processes, know the information about the Server NWDAF and inform Server NWDAF directly. The procedure is the following:

0.	Server NWDAF registers into NRF about the Federated Learning procedure with the following parameters:

-	Federated Learning (FL) Correlation ID

-	Analytics ID

-	Client NWDAF(s) information

NOTE:	FL Correlation ID is used to identify a specific FL procedure. For example, an server NWDAF or a client NWDAF can join different FL procedures at the same time, then when they receive messages or data from other NWDAF, they have to know the message or data is for which FL procedure.

When a server NWDAF starts a FL procedure, it registers the FL procedure in the NRF with FL Correlation ID, Analytics ID. When later a client NWDAF wants to join a FL dynamically, e.g. it wants to update its local model using global information, it will query NRF if there is an ongoing FL for the analytics ID. Then NRF will provide the server NWDAF ID and FL Correlation ID to the client NWDAF, then the client NWDAF can contact the server NWDAF to join the FL procedure. With the FL correlation ID, the server NWDAF knows which FL procedure the client NWDAF wants to join and which model it should provide to the client.

Editor's note:	It is FFS if server NWDAF registers/updates client NWDAFs info into NRF

1.	If the information about the Server NWDAF and the corresponding FL procedure is known via NRF, new Client NWDAF(s) inform Server NWDAF about their interoperability and availability to join the Federated Learning process during the Federated Learning training processes.

2.	Before starting next round of training, Server NWDAF selects Client NWDAF(s) from NWDAFs 1 to N+X based on the updated information of the Client NWDAF(s). The procedure is the same as steps 1-5 in figure 6.51.2.1-1.

6.51.2.3.2	Server NWDAF Gets the Information of New Client NWDAF(s) via NRF

Figure 6.51.2.3.2-1: Procedure for dynamic discovery of new NWDAFs in Federated Learning execution phase when the information about the Server NWDAF is unknown at the new Client NWDAF(s)

Client NWDAFs 1 to N are selected by Server NWDAF for participating the current round of Federated Learning. Client NWDAFs N+1 to N+X, which are the new ones, have the capability to join in the next rounds of training processes.

Server NWDAF gets the information of the new Client NWDAF(s) dynamically via NRF by either subscribing to the event that a new Client NWDAF registers or discovering NRF when it needs to reselect Client NWDAFs.

6.51.3	Impacts on services, entities and interfaces

NWDAF:

-	Register FL procedure information into NRF dynamically during the Federated Learning processes

-	Selection of Client NWDAF(s)

-	Monitoring Client NWDAF status updates

-	Dynamic discovery of new Client NWDAF(s)

NRF:

-	Server NWDAF and Client NWDAF(s) registration for the Federated Learning procedure

-	Registers FL procedure information dynamically during the Federated Learning processes

-	Receive and notify information about Client NWDAF(s) updates to Server NWDAF

6.52	Solution #52: FL training update to NWDAF containing AnLF from NWDAF containing MTLF

6.52.1	Description

This solution is proposed for KI#8 to support the Federated Learning procedure between different NWDAFs containing MTLF and provide Federated Learning training updates from NWDAF containing MTLF (with FL aggregation capability/ in the role of FL server) to NWDAF containing AnLF.

6.52.2	Procedures

Procedure for Federated Learning training updates from NWDAF containing MTLF to NWDAF containing AnLF is described in Figure 6.52.2-1.

Figure 6.52.2-1: Procedure for FL training update to NWDAF containing AnLF from NWDAF containing MTLF

Editor's note:	The aspects related to trained ML model(s) sharing should be aligned with Key Issue #5.

0.	NWDAF containing AnLF discovers one or more instances of NWDAF containing MTLF via the NRF as illustrated in steps 0a, 0b, 0c.

1.	FL training between NWDAF containing MTLF (with Federated aggregation capability) and NWDAF containing MTLF (with Federated participation capability) is performed as described in Solution 21 Figure 6.21.2.3-1 or Solution 23 Figure 6.23.2-1.

2.	The NWDAF containing MTLF sends Nnwdaf_MLmodelProvision_Notify message with Analytics ID(s), ML model ID(s), ML model file address, ML model serialization format, Training Accuracy metric per ML model ID. The training accuracy metrics indicates the ML model accuracy when the NWDAF containing MTLF performs training using training dataset.

3.	[conditional] NWDAF containing AnLF sends Nnwdaf_MLModelTrainingUpdate_Subscribe message to the NWDAF containing MTLF with Analytics ID(s), ML model ID(s), Base Accuracy metric, Notification Correlation ID. The same ML model ID(s) as provided in step 2 are included. The Base Accuracy metric is an accuracy metric determined by the NWDAF containing AnLF using the dataset from live network. The Base Accuracy metric is provided by the NWDAF containing AnLF to notify the NWDAF containing MTLF, when the same ML model as in step 2 or a new ML model (for a given Analytics ID) is available with training accuracy higher than Base Accuracy metric.

Editor's note:	The Accuracy metric used depends on the conclusion of KI#1.

4.	[conditional] NWDAF containing MTLF sends Nnwdaf_MLModelTrainingUpdate_Notify message with Analytics ID(s), ML model ID(s), Training Accuracy metric. The ML model ID(s) included in the message may be the same re-trained ML model as provided in step 2 with new training accuracy higher than the Base Accuracy metric provided in step 3 or the ML model ID(s) may be a new trained ML model available for the Analytics ID(s) provided in step 3 with training accuracy level higher than the Base Accuracy metric.

5.	[conditional] NWDAF containing AnLF decides if it wants to use the ML model ID provided in step 4.

6.	[conditional] NWDAF containing AnLF sends Nnwdaf_MLModelProvision_Request with Analytics ID(s), ML model ID(s) provided in step 4.

7.	[conditional] NWDAF containing MTLF sends Nnwdaf_MLModelProvision_Response with Analytics ID(s), ML model ID(s), ML model file address, Training Accuracy metric.

6.53	Solution #53: Horizontal Federated Learning with Multiple NWDAF

6.53.1	Description

This is a solution for the Key Issue#8: Supporting Federated Learning in 5GC.

As shown in Figure 6.53.1-1, there are several NWDAF (containing MTLF) distributed in an operator, where each NWDAF (containing MTLF) is responsible for AI model training based on collected data per certain area.

It looks necessary that various data from wide area is needed to train a better ML model for operator however it is difficult for NWDAF (containing MTLF) to collect all the raw data from distributed data source in different areas. For example, as in shown in figure 1 below, the raw data per serving area is stored in each local NWDAF (containing MTLF).

Figure 6.53.1-1: Distributed NWDAF(containing MTLF) deployment in a PLMN

It is proposed to utilize Horizontal Federal ML learning to train a ML model by retrieving the useful information from the raw data from distributed data source over wide area but not require to collect/store all the raw data from distributed data source over the wide area.

The main idea of Horizontal Federated Learning is to build machine-learning models based on data sets that are distributed in different network functions. As shown in Figure 6.53.1-1, NWDAF (containing MTLF1) is chosen as coordinator and the other NWDAF (containing MTLF) are chosen as client.

A client NWDAF (containing MTLF) locally trains the local ML model with its own data and shares it to the coordinator NWDAF (containing MTLF). With local ML models from different Client NWDAF (containing MTLF):

1)	If the coordinator NWDAF (containing MTLF) is with the capability of FL server, it aggregates the local ML models into a global or optimal ML model or ML model parameters and send them back to client NWDAF(containing MTLF).

2)	If the coordinator NWDAF (containing MTLF) is only with the capability of FL client, it will find out a NWDAF(containing MTLF) support FL server, who will be responsible to aggregate the local ML models into a global or optimal ML model or ML model parameters and send them back to client NWDAF(containing MTLF).

The proposed solution is composed the following aspects:

-	Registration and discovery of multiple NWDAF (containing MTLF) with capability of Horizontal Federated Learning

-	Horizontal Federated Learning training procedure

6.53.2	Procedures

6.53.2.1	Procedure for registration multiple NWDAF (containing MTLF) with capability of Horizontal Federated Learning

Figure 6.53.2.1-1: Registration and discovery for Horizontal Federated Learning

1-3.	The MTLF with FL capability registers its NF profile (Address of MTLF, Supported Analytics ID(s), Horizontal Federated Learning Indication(FL client, FL server), Serving Area, preferred Time Period for FL) into NRF.

The Serving Area indicates the area where MTLF is to collect the raw data.

Preferred Time Period for FL indicates when the MTLF prefers to participate in Horizontal Federated Learning, e.g. based on local configuration.

6.53.2.2	Horizontal Federated Learning Training triggered by MTLF with FL Server capability

Figure 6.53.2.2-1: Horizontal Federated Learning Training triggered by MTLF with FL Server capability

1.	The AnLF subscribes to a trained ML Model associated with a Analytics ID by invoking the Nnwdaf_MLModelProvision(Analytics ID, ML model Filter(S-NSSAI, Area of Interest)) service operation.

The parameter Area of Interest provided by ANLF (as listed in clause 6.2A.2, TS 23.288 [5]) requests the MTLF1 with FL server to train the ML model associated with the Analytics ID based on the collected data over the Area of Interest.

As the MTLF1 with FL server lacks of data over the Area of Interest requested by the AnLF, it determines that training based on Horizontal Federated Learning is needed and is to discover multiple MTLF with FL client which have the data of the area of interest and could be used for Horizontal Federated Learning via the NRF.

2-5.	The MTLF1 with FL server acting as coordinator is to discover multiple MTLF with FL client which could be used for Horizontal Federated Learning clients via the NRF by invoking the Nnrf_NFDiscovery_Request (an Analytics ID, Horizontal Federated Learning Indication, Serving Area, Time Period of Interest) service operation.

The NRF notifies the MTLF1 with FL server with the information of multiple MTLF with FL client, which is used to kick off Horizontal Federated Learning procedure as defined in 6.53.5 in details.

6.	The MTLF1 with FL server notifies the AnLF with the trained ML model information (a file address of the trained ML model) by invoking Nnwdaf_MLModelProvision_Notify service operation.

6.53.2.3	Horizontal Federated Learning Training triggered by MTLF only with FL Client capability

Figure 6.53.2.3-1: Horizontal Federated Learning Training triggered by MTLF only with FL Client capability

1.	The AnLF subscribes to a trained ML Model associated with a Analytics ID by invoking the Nnwdaf_MLModelProvision(Analytics ID, ML model Filter(S-NSSAI, Area of Interest)) service operation.

The parameter Area of Interest provided by ANLF (as listed in clause 6.2A.2, TS 23.288 [5]) requests the MTLF0 with FL Client to train the ML model associated with the Analytics ID based on the collected data over the Area of Interest.

As the MTLF0 with FL Client lack of data over the Area of Interest requested by the AnLF, it determines that training based on Horizontal Federated Learning is needed.

In addition, as the MTLF0 with FL Client cannot act as FL server and consequently query NRF to find out the MTLF1 with FL server, who is requested by the MTLF0 with FL Client to kick off Horizontal Federated Learning procedure.

Step 2-5 are same with step2-5 as defined in clause 6.53.3.

6.	The the MTLF1 with FL server notifies the MTLF0 with FL Client with the trained ML model information (a file address of the trained ML model).

The MTLF0 with FL Client notifies the AnLF with the trained ML model information (a file address of the trained ML model) by invoking Nnwdaf_MLModelProvision_Notify service operation.

6.53.2.4	Horizontal Federated Learning training procedure

Figure 6.53.2.4-1: Horizontal Federated Learning training procedure

1.	Based on the response from NRF as specified in step 7, clause 6.53.1.2, the MTLF1 with FL server acting as coordinator selects multiple MTLF with FL client to kick off Horizontal Federated Learning procedure by sending a request to the selected MTLF with FL client including parameters such as analytics ID, model instance ID for the FL trained model, initial ML model, data type list, maximum response time window, model filter information for each MTLF with FL clients to retrieve local ML model, the number of iterations for each MTLF with FL client per training round, etc.

2.	Each selected each MTLF with FL client collects its local raw data with the mechanism defined in clause 6.2, TS 23.288 [5].

3.	During Federated Learning training procedure, the selected MTLF with FL client trains the retrieved ML model with its local raw data over the its own serving area and reports the results of ML model training to the MTLF1 with FL server acting as coordinator.

4.	The MTLF1 with FL server acting as coordinator aggregates all the local ML model training results retrieved at step 3 to update the ML model.

5-6.	The MTLF1 with FL server acting as coordinator sends the aggregated ML model information (updated ML model) to the selected MTLF with FL client, which is used by each selected MTLF with FL client to update the local own ML model in itself).

NOTE:	The steps 3-6 should be repeated until the training termination condition is reached e.g. based on maximum number of iterations configured by the MTLF1 with FL server acting as coordinator.

Editor's note:	Whether and how to standardize the interaction between the MTLF1 with FL server acting as coordinator and the MTLF with FL client is FFS.

6.53.3	Impacts on existing nodes and functionality

MTLF with FL client:

-	register the "FL capability, Serving Area, Supported Time Period for FL " in NF profile to NRF.

-	support local ML training based on instruction from the MTLF acting as coordinator.

-	support to report the trained interim ML model to NWDAF(MTLF) acting as coordinator

-	support MTLF with FL Server discovery procedure.

MTLF with FL server acting as coordinator:

-	register the "FL capability, Serving Area, Supported Time Period for FL " in NF profile to NRF.

-	support MTLF with FL client discovery procedure.

-	support to send FL request to MTLF with FL client.

-	support to aggregate the interim ML model from MTLF with FL client.

6.54	Solution #54: Finer granular location information based on LCS input data

6.54.1	Description

As described in the TS 23.288 [5], currently the preferred granularity of location information is either on TA level or cell level, however, below use case demands more precious location information.

-	Measuring the number of viewers who see outdoor advertisements: In order to measure the users who see the advertisement in a specific location, statistical information of a specific location is required to estimate the viewers of an advertisement.

-	Inference of taste and recommendation based on visited places: it is important to consider the location information before sending the recommendation. The exact location is crucial for deriving the analysis for location-based services (for example, mydaiz service provided by NTT DOCOMO).

In this solution, it is defined location granularity and location accuracy granularity as:

-	Location granularity:

  - -	TA level;

  - -	Cell level;

  - -	Geographic Location and Dispatchable Location as described in clause 4.2, TS 22.071 [6].

-	Location accuracy granularity:

  - -	Horizontal Accuracy, Vertical Accuracy and Dispatchable Location accuracy as described in clause 4.3, TS 22.071 [6].

The location information can be UE specific or group of UEs in specific location, in order for NWDAF to provide analytic, therefore, we propose that NWDAF will act as an LCS client and extract the location from LCS system (GMLC). Based on the input data from LCS, the NWDAF determine the finer granular location statistical or prediction information and provides to the consumer (such as AF or 5GC NF(s)).

Editor's note:	More information about input and output data for the finer granular location statistical or prediction information is required.

6.54.2	Procedures

Figure 6.54.2-1: Procedure for a precious location collection from LCS system.

1.	The NF consumer sends the Nnwdaf_AnalyticsInfo to the NWDAF. The Analytics ID is set to either existing UE location analytic ID (e.g., UE Communication Analytics) or a new precious location analytic, the Target of Analytics Reporting is set according to clause 6.5.1, TS 23.288 [5] and Analytics Filter Information include Location granularity and Location accuracy granularity. The NF can request statistics or predictions or both and can provide a time window.

  - 2-3.	NWDAF selects the LCS NF (GMLC) with UE location information as described in clause 5, TS 23.273 [13] and collects the UE location information from LCS system as described in clause 6.1, TS 23.273 [13].

4.	Based on the collected data, the NWDAF determine the UE location statistics and/or predictions and provides to the consumers.

6.54.3	Impacts on Existing Nodes and Functionality

The NWDAF need to expose the new analytic ID with new filter information, or enhancements are needed to the existing analytic ID with new filter information such as Location granularity and Location accuracy granularity. Furthermore, NWDAF needs to collect the input data from the LCS system e.g., NWDAF will act as a LCS client and gets the input data.

6.55	Solution #55: location information with finer granularity in horizontal and vertical directions

6.55.1	Description

The UE location information provided to NWDAF is only TA/cell granularity. However, NWDAF needs horizontal accuracy and the vertical accuracy with finer granularity than TA or cell level. For example, in a large interchange bridge, the horizontal accuracy and the vertical accuracy with TA/cell granularity of the UE location information is not enough to produce an accurate analytics or advice.

For accurate analytics or advice, the granularity of location information provided to NWDAF should be finer than TA/cell level. This solution proposes a procedure to enhance the NWDAF with finer granularity of location information and add extra input data to complete itself.

6.55.2	Input Data

This solution helps the consumer NF to obtain UE location analytics also the NWDAF to obtain location information with finer granularity than TA/cell. Considering scenarios like high-rise buildings and large interchange bridges, it is beneficial to distinguish the UE velocity, granularity choice and UE location in horizontal and vertical directions.

The location related data are defined in Table 6.55.2-1.

Table 6.55.2-1: Input data for location information with finer granularity in horizontal and vertical directions

| Information | Source | Description |
| --- | --- | --- |
| Granularity choice (NOTE 1) | Consumer NF | Location information with TA/cell granularity or finer granularity than TA/cell. |
| > horizontal direction |  | Location information in horizontal direction with TA/cell granularity or finer granularity than TA/cell. |
| > vertical direction |  | Location information in vertical direction with TA/cell granularity or finer granularity than TA/cell. |
| UE velocity | LCS | UE velocities in horizontal and vertical directions. |
| > horizontal direction |  | UE velocity in horizontal direction. |
| > vertical direction |  | UE velocity in vertical direction. |
| UE locations | AMF, LCS (NOTE 2) | UE positions |
| > UE location (NOTE 3) |  | TA or cells that the UE enters or UE location with finer granularity than TA/cell. |
| >> horizontal direction |  | TA or cells that the UE enters or UE location with finer granularity than TA/cell in horizontal direction. |
| >> vertical direction |  | TA or cells that the UE enters or UE location with finer granularity than TA/cell in vertical direction. |
| > Timestamp |  | A time stamp when the AMF detects the UE enters this location |

NOTE 1:	After adding location information with finer granularity than TA/cell, there are 3 different data granularities can be chosen: TA level, cell level or finer granularity than TA/cell level. Besides, location information in horizontal and vertical directions can have different granularities.

Editor's note:	UE location needs to be in the form of some area to enable related analytics. It is FFS how areas can be defined with finer granularity than TA/cell level.

NOTE 2:	If granularity choice is TA/cell level, the source of UE location related data is AMF; If granularity choice is finer granularity than TA/cell level, the source of UE location related data is LCS architecture.

NOTE 3:	Because there can be different granularities in different directions, UE location in 2 directions also can have different granularities. If granularity choices in horizontal and vertical directions are both TA/cell level, UE locations in horizontal and vertical directions are the same TA/cell. Otherwise, UE locations in horizontal and vertical directions are different, for example, UE location in horizontal direction is TA/cell and UE location in vertical direction is with finer granularity than TA/cell.

6.55.3	Output Data

The output are defined in Table 6.55.3-1 and Table 6.55.3-2.

Table 6.55.3-1: Location information with finer granularity in horizontal and vertical directions statistics

| Information | Description |
| --- | --- |
| Time slot start | Time slot starts within the Analytics target period |
| Duration | Duration of the time slot |
| UE location | Observed location statistics |
| > UE location | TA or cells which the UE stays |
| >> horizontal direction | TA or cells which the UE stays or UE location with finer granularity than TA/cell in horizontal direction |
| >> vertical direction | TA or cells which the UE stays or UE location with finer granularity than TA/cell in vertical direction |

Table 6.55.3-2: Location information with finer granularity in horizontal and vertical directions predictions

| Information | Description |
| --- | --- |
| Time slot start | Time slot starts within the Analytics target period |
| Duration | Duration of the time slot |
| UE location | Observed location statistics |
| > UE location | TA or cells which the UE stays |
| >> horizontal direction | TA or cells which the UE stays or UE location with finer granularity than TA/cell in horizontal direction |
| >> vertical direction | TA or cells which the UE stays or UE location with finer granularity than TA/cell in vertical direction |
| > Confidence | Confidence of this prediction |

6.55.4	Procedures

Figure 6.55.4-1 illustrates the procedure for location information with finer granularity in horizontal and vertical directions.

Figure 6.55.4-1: Procedure for location information with finer granularity in horizontal and vertical directions

1.	The consumer NF sends a request (Analytics ID = finer granularity in 2 directions, Target of Analytics Reporting = UE id, Analytics Filter Information = AOI) to the NWDAF for analytics information using either the Nnwdaf_AnalyticsInfo or Nnwdaf_AnalyticsSubscription service. The request carries what kind of granularities the consumer asks for in horizontal and vertical directions and two directions can have different granularity demand.

2a.	If the granularity of UE location in horizontal or vertical direction is finer than TA/cell, NWDAF collects input data related to UE location from the LCS.

2b.	If the granularity of UE location in horizontal or vertical direction is TA/cell level, the UE location data is collected from the AMF.

3.	The NWDAF derives requested analytics.

4.	The NWDAF provides requested finer granularity in 2 directions analytics to the NF, using either the Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1. The details for the analytics provided by NWDAF are defined in output analytics.

6.55.5	Impacts on Existing Nodes and Functionality

NWDAF:

-	The NWDAF can collect input data related to UE location with finer granularity than TA/cell level from the LCS.

-	The NWDAF can support the analytics ID for finer granularity in 2 directions.

6.56	Solution #56: PSAP resolution with finer granularity of location information

6.56.1	Description

This solution addresses Key issue #9: Enhancement of NWDAF with finer granularity of location information. This solution proposes a use case about PSAP (Public Safety Answering Point) resolution for emergency services with finer granularity of location information.

Editor's note:	It is FFS if this use case requires analytics about UE location or only the UE location when emergency communication was initiated from that UE. Is it realistic that a UE can start emergency communication, but the UE location cannot be determined?

6.56.2	Functional description

At a high level, the functional description of this solution is as follows:

-	UE initiates an emergency session request with including emergency URI and Cell Global Identification (CGI) with existing procedure.

-	If required, IMS network access to the NWDAF to retrieve the PSAP destination with the estimated UE's location with finer granularity than Cell/TA level

-	The IMS network uses the routing information returned by the NWDAF to route the emergency session request towards the appropriate PSAP.

Input data (example):

-	UE ID

-	Cell Global Identification (CGI)

-	UE location info (if available)

Output data (example):

-	PSAP destination

6.56.3	Procedures

Figure 6.56.3-1: Emergency Session Establishment procedure with using NWDAF

6.56.4	Impacts on services, entities and interfaces

Existing Emergency service procedure will be updated to acquire the PSAP destination with the finer granularity of location info than Cell ID/TA level.

NWDAF:

-	The NWDAF collects and analysis data from LCS and the other NFs to determine UE location and routing information.

-	The new Analytic ID supporting the PSAP resolution use case needs to be supported.

Editor's note:	The name of new analytic ID is FFS.

Editor's note:	The existing analytic ID e.g., UE mobility, can be used to address the PSAP resolution use case.

6.57	Solution #57: NWDAF determines granularity when the consumer requests finer granularity location information

6.57.1	Description

This solution mainly addresses Key issue #9: Enhancement of NWDAF with finer granularity of location information.

In the current eNA specification, the NWDAF consumer delivers its requirement for data collection, analytics, prediction, and so on. On the one hand, many existing analytics IDs, such as Service Experience, UE Mobility, and Dispersion Analytics, support NWDAF consumers by providing the preferred granularity of location information. Some potentially new analytics IDs may also need to provide preferred granularity of location information.

On the other hand, the consumers can also request a parameter like LCS QoS to express a more specific requirement when collecting UE location information. The NWDAF can in return to reflect QoS level to the consumers, helping consumers judge the situation of the current interaction link between NWDAF and LCS architecture.

This solution will introduce more granularity options and LCS QoS and by taking analytics ID = UE mobility as an example, which can also be applicable for the analytic ID where the consumer includes the preferred granularity of location information in the request message.

The consumer of these analytics may indicate in the request:

-	Analytics ID = "UE Mobility".

-	Target of Analytics Reporting: a single UE (SUPI) or a group of UEs (an Internal Group ID);

-	Analytics Filter Information optionally containing:

-	Area of Interest (AOI): restricts the scope of the UE mobility analytics to the provided area;

-	Visited Area(s) of Interest (visited AOI(s)): additional filter to only consider UEs that are currently (i.e. now) in the "AOI" and had previously (i.e. in the "Analytics target period") been in at least one of the Visited AOI(s). If this parameter is provided, the Analytics target period shall be in the past (i.e. supported for statistics only);

-	An Analytics target period indicates the time period over which the statistics or predictions are requested;

-	Optionally, maximum number of objects;

-	Preferred level of accuracy of the analytics;

-	Preferred order of results for the time slot entries: ascending or descending time slot start;

-	Optionally, preferred granularity of location information: TA level, cell level, or different granularity options. The different granularity may be mapped into different accuracy requirements in the LCS;

-	Optionally, preferred LCS QoS: This information is included when the preferred granularity of location information is finer than TA/cell level; and

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

6.57.2	Input Data

If the finer granularity location information is required, the NWDAF retrieves location information from LCS architecture. The preferred granularity can be mapped into the accuracy of the location operation and the preferred LCS QoS can be mapped into LCS QoS.

The detailed information collected by the NWDAF could be MDT data from OAM, network data from 5GC and/or service data from AFs:

-	UE mobility information from OAM is UE location carried in MDT data;

-	Network data related to UE mobility from 5GC is UE location information, UE location trends or UE access behaviour trends, as defined in the Table 6.57.2-1;

Table 6.57.2-1: UE Mobility information collected from 5GC

| Information | Source | Description |
| --- | --- | --- |
| UE ID | AMF | SUPI |
| UE locations (1..max) | AMF | UE positions |
| >UE location | (NOTE 2) | Geographical location where the UE stays with a finer granularity (NOTE 1) |
| >LCS QoS | (NOTE 2) | Achieved LCS QoS during location operation |
| >Timestamp | (NOTE 2) | A time stamp when the location operation happens |
| Type Allocation code (TAC) | AMF | To indicate the terminal model and vendor information of the UE. The UEs with the same TAC may have similar mobility behaviour. The UE whose mobility behaviour is unlike other UEs with the same TAC may be an abnormal one. |
| Frequent Mobility Registration Update | AMF | A UE (e.g. a stationary UE) may re-select between neighbour cells due to radio coverage fluctuations. This may lead to multiple Mobility Registration Updates if the cells belong to different registration areas. The number of Mobility Registration Updates N within a period M may be an indication for abnormal ping-pong behaviour, where N and M are operator's configurable parameters. |
| UE access behaviour trends | AMF | Metrics on UE state transitions (e.g., access, RM and CM states, handover). |
| UE location trends | AMF | Metrics on UE locations. |
| NOTE 1: UE location includes either the last known location or the current location, under the conditions defined in Table 4.15.3.1-1 in TS 23.502 [3]. NOTE 2: Which NF interacts with the NWDAF is coordinated with eLCS study. |  |  |

-	Service data related to UE mobility provided by AFs is defined in the Table 6.57.2-2;

Table 6.57.2-2: Service Data from AF related to UE mobility

| Information | Description |
| --- | --- |
| UE ID | Could be external UE ID (i.e. GPSI) |
| Application ID | Identifying the application providing this information |
| UE trajectory (1..max) | Timestamped UE positions |
| >UE location | Geographical location where the UE stays with a finer granularity |
| >LCS QoS | Achieved LCS QoS during location operation |
| >Timestamp | A time stamp when the location operation happens |
| NOTE: The application ID is optional. If the application ID is omitted, the collected UE mobility information can be applicable to all the applications for the UE. |  |

6.57.3	Output Analytics

The NWDAF provides UE mobility analytics to consumers under the preferred granularity and preferred LCS QoS and so on. The analytics results provided by the NWDAF could be UE mobility statistics as defined in table 6.57.3-1, UE mobility predictions as defined in Table 6.57.3-2:

Table 6.57.3-1: UE mobility statistics

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, SUPI. |
| Time slot entry (1..max) | List of time slots during the Analytics target period |
| > Time slot start | Time slot start within the Analytics target period |
| > Duration | Duration of the time slot |
| > UE location (1..max) | Observed location statistics |
| >> UE location | Geographical location where the UE stays with a finer granularity |
| >> LCS QoS | Achieved LCS QoS during location operation |
| >> Ratio | Percentage of UEs in the group (in the case of a UE group) |

Table 6.57.3-2: UE mobility predictions

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, or SUPI. |
| Time slot entry (1..max) | List of predicted time slots |
| >Time slot start | Time slot start time within the Analytics target period |
| > Duration | Duration of the time slot |
| > UE location (1..max) | Location prediction |
| >> UE location | a finer granularity location where the UE or UE group may move into |
| >> LCS QoS | LCS QoS achieved in the future period |
| >> Confidence | Confidence of this prediction |
| >> Ratio | Percentage of UEs in the group (in the case of a UE group) |

Editor's note:	UE location needs to be in the form of some area to enable related analytics. It is FFS how areas can be defined with finer granularity than TA/cell level.

6.57.4	Procedures

Figure 6.57.4-1 depicts a procedure for finer location information analytics service provided by NWDAF.

Figure 6.58.4-1: Parameters enhancement when the finer location information is requested

1.	The consumer requests finer location information statistics or predictions. The parameters like preferred granularity, preferred LCS QoS, and other parameters are carried in the request message. The preferred LCS QoS is included only when the preferred granularity is finer than TA/cell level.

2.	The NWDAF collects mobility information from LCS architecture. The LCS architecture returns the UE location information and achieved LCS QoS to the NWDAF.

3.	The NWDAF initiates location information analysis under the preferred granularity and preferred LCS QoS.

4.	The NWDAF provides the statistics or the prediction to the consumers in the output analytics. The NWDAF can also predict the achieved LCS QoS in the future time.

6.57.5	Impacts on Existing Nodes and Functionality

NWDAF:

-	The NWDAF supports more granularities in the preferred granularity of location information and preferred LCS QoS when the consumers request finer location information.

LCS:

-	The LCS architecture supports providing LCS QoS and finer location information to NWDAF.

6.58	Solution #58: Supporting UE mobility analytics with finer granularity than TA/cell

6.58.1	Description

If the NWDAF requires to provide UE mobility analytics with finer granularity than TA/cell then the NWDAF can interface with the GMLC/AMF to obtain location information using the LCS procedures. In such scenario the NWDAF interface with a GMLC to obtain location information for a UE. Another approach is that the NWDAF acts as a GMLC client and obtain location information directly from the AMF.

The UE mobily analytics output include as additional analytic output the UE location in geographical coordinates and/or local coordinates expressed as a shape as defined in TS 23.032 [14].

In addition, as part of the support of Location Services work, specified in 3GPP TS 23.273 [13], a consumer may request a deferred location request requesting to be notified of a location of a UE when specific events associated to a target UE take place. The following types of events are defined:

a)	UE availability: Any event in which the 5GCN has established a contact with the UE. This event is considered to be applicable when the UE is temporarily unavailable due to inaction by the user, or for temporarily loss of radio connectivity or IMSI detach and so on

b)	Area: An event where the UE enters, leaves or remains within a pre-defined geographical area.

c)	Periodic Location: An event where a defined periodic timer expires in the UE and activates a location report.

d)	Motion: An event where the UE moves by more than some predefined straight line distance from a previous location. The motion event may be reported one time only, or multiple times.

The UE mobility analytics provided by the NWDAF can be additionally enhanced to collect location information for one or more UEs (or any UE) using the deferred location events supported by the eLCS architecture. Such scenario is useful if the consumer requires to be aware of the UE mobility trends in a target area. For example, a consumer may require to be aware of mobility analytics of UEs that are in motion in a target area.

6.58.2	Procedures

If a consumer requires UE mobility analytics with finer granularity than TA/cell then the following apply.

Figure 6.58.2-1: Supporting UE mobility analytics taking into account mobility trends.

1.	A Consumer requires UE mobility analytics taking into account UE mobility trends or specific pre-defined geographical area of a UE or group of UEs or UEs in a target area.

2.	Consumer sends a request for analytic including the analytic ID and as analytic filters.

-	A target area.

-	Motion Event Criteria specifying a threshold linear distance that the UE must make before the UE provides a location.

3.	The NWDAF requests location reports from the GMLC/AMF using eLCS location procedures as described in 3GPP TS 23.273 [13].

4.	NWDAF derives UE mobility analytics

5.	Analytics are reported to the consumer NF.

The following input data are required by the NWDAF to measure UE mobility analytics for specific UE mobility trends in addition to the input data defined in clause 6.7.2.2 of 3GPP TS 23.288 [5].

Table 6.58.2-1: Location measurement from LMF (via AMF)

| Information | Source | Description |
| --- | --- | --- |
| UE location estimate | GMLC/AMF | The estimated location of the UE determined by the LMF |
| Indication of motion event occurence | GMLC/AMF | Provides an indication that the UE has moved a requested linear threshold distance |
| Timestamp | GMLC/AMF | Time stamp of location estimation |

The UE mobility analytics output to a consumer include the following, in addition to the analytic output data defined in clause 6.7.2.3 of 3GPP TS 23.288 [5].

Table 6.58.2-2: UE mobility statistics

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, SUPI. |
| Time slot entry (1..max) | List of time slots during the Analytics target period |
| > Time slot start | Time slot start within the Analytics target period |
| > Duration | Duration of the time slot |
| > UE location (1..max) | Observed location statistics |
| >> UE location | A list of TA or cells or geographical/local coordinates as described in 3GPP TS 23.032 [14] reported by the UE |
| >> UE route | Locations reported by the UE between motion events |
| …..>> UE mobility trends | Distance travelled within reported location(s) |
| >> Ratio | Percentage of UEs in the group (in the case of a UE group) |

Table 6.58.2-3: UE mobility predictions

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, or SUPI. |
| Time slot entry (1..max) | List of predicted time slots |
| >Time slot start | Time slot start time within the Analytics target period |
| > Duration | Duration of the time slot |
| > UE location (1..max) | Predicted location prediction during the Analytics target period |
| >> UE location | TA or cells or geographical/local coordinates as described in 3GPP TS 23.032 [14] where the UE may move into |
| >> UE route | Predicted route of the UE |
| … >> UE mobility trends | Distance travelled within reported location(s) |
| >> Confidence | Confidence of this prediction |
| >> Ratio | Percentage of UEs in the group (in the case of a UE group) |

Editor's note:	UE location needs to be in the form of some area to enable related analytics. It is FFS how areas can be defined with finer granularity than TA/cell level.

6.58.3	Impacts on services, entities and interfaces

-	NWDAF supporting eLCS SBI to retrieve UE location information from GMLC/AMF

6.59	Solution #59: Enhancement of NWDAF with location accuracy prediction

6.59.1	Functional Description

6.59.1.1	General

This solution addresses the KI "Key Issue #9: Enhancement of NWDAF with finer granularity of location information".

Editor's note:	This paper is only focused on NWDAF enhancement. For LCS related part coordination with eLCS_ph3 is needed and eNA_ph3 will start work only if eLCS_ph3 makes progress with stable outcome.

TS 23.273 [13] defines LCS QoS to have the following attributes: (1) Location Accuracy, (2) Response time and (3) QoS Class. Among these, the QoS Class attribute gives the requirement on the other attributes like accuracy or response time. It could take as values Best Effort, Multiple QoS Class or Assured. While the Best Effort class is the least stringent one, the other two require the LMF to determine how accurate its location estimate is.

For example:

-	in case of "Multiple QoS Class", if the accuracy is less than what is required, the LMF selects an appropriate localization method (e.g. with more TRP measurements) and re-runs the location estimate in order to reach the required accuracy.

-	in case of "Assured" QoS Class, the procedure fails if the accuracy required is not met.

To provide a real-time estimate of location accuracy without need for benchmark/ground truth location, we propose a data analytics-based solution running in the NWDAF with a supervised Machine Learning model (e.g. Neural Network).

A training should be first performed on the selected ML model with labelled data, i.e. input data with corresponding output data:

-	Input data: positioning estimate and optionally: used positioning method, assistance data, UE ID, Area, see Table 6.59.1-1.

-	Output data: corresponding accuracy (calculated with benchmark method such as the output of MDT), see Table 6.59.1-2

Once the model is trained, there is no more need for ground truth (e.g. GPS information). This model is then subsequently used by the LMF to determine if its location estimate meets the accuracy required and take appropriate action. To enable this, the LMF makes use of a Service Based API that is exposed by the NWDAF.

The input data for the Location Accuracy analytics is shown in Table 6.59.1-1

Table 6.59.1-1: Data collection for "Location accuracy" analytics

| Information | Source | Description |
| --- | --- | --- |
| UE location | LMF | UE location as estimated by the LMF with the selected localization method. |
| UE ID | LMF | UE identifier |
| Location method/Assistance Data | LMF | Selected location method and corresponding parameters. |
| Area | LMF | The region of interest where the UE is located (e.g. could be list of cell IDs). |

The Location Accuracy service output is defined in Table 6.59.1-2.

Table 6.59.1-2: Output analytics for "Location accuracy"

| Information | Description |
| --- | --- |
| Location accuracy | Predicted location accuracy (could be in % or in unit meters) |
| Applicable Area | A list of TAIs or Cell IDs as well as below cell level location information that the analytics applies to. |
| Confidence | Confidence of the prediction. |

Editor's note:	If the proposed Analytics ID can provide location accuracy prediction is FFS.

6.59.2	Procedure

Figure 6.59.2-1: Location accuracy prediction and usage

Pre-condition: NWDAF has a trained supervised ML model for predicting location accuracy.

1.	LCS Client requests to LMF the UE location with QoS Class set as either "Multiple QoS Class" or "Assured" alongwith the location accuracy required. The expected behaviour from the LMF is as given in clause 6.59.2.

2.	The LMF initiates the LCS session as given in TS 23.273 [13] and derives the UE location estimate.

3.	The LMF queries the NWDAF for the location accuracy giving as inputs the corresponding Analytics ID . Other inputs could be - location estimate, UE positioning method/assistance data and area (cell ID/TAI).

NOTE 1:	Both Nnwdaf_AnalyticsSubscription and Nnwdaf_AnalyticsInfo services can be used to implement step 3. above. The Nnwdaf_AnalyticsSubscription service can be used by a service consumer to receive notifications about location accuracy, e.g., when a change is detected.

4.	The NWDAF uses the corresponding model trained in the precondition and provides the location accuracy as the output to the LMF.

5.	LMF compares the location accuracy received in step 3 with the required location accuracy received in step 1 from the LCS Client.

6.	If the required location accuracy is not met, the LMF takes subsequent actions like re-executing the LCS procedure with more stringent parameters (for example: higher periodicity measurements).

7.	The LMF returns the location estimate with the required accuracy.

NOTE 2:	The offline training gives the ML model the ability to predict the location accuracy without the need for a reference location estimate. This is made possible by the extensive training done using various accuracy/reference values, which result in the computing of weights in the ML model. Such a trained model can then be used for an inference/prediction of the location accuracy in this step 3.

6.59.3	Impacts on services, entities and interfaces

NWDAF

-	Supports a new Analytics ID for determining Location Accuracy

-	Supports subscription from NFs like LMF to make use of this Analytics

-	Supports data collection from LMF to generate the output of this Analytics

6.60	Solution #60: Interactions with MDAF

6.60.1	Description

MDAS/MDAF functionality enables a service consumer to obtain management data analytics, and NWDAF can be one of such service consumers.

For Oberserved Service Experience analysis, NWDAF can leverage the service experience analytics output from MDA defined in TS 28.104 [15] clause 8.4.2.1.3 in addition to the input as described in TS 23.288 [5] clause 6.4.2. And also, analytics output for energy saving analysis, such as StatisticsOfCellsEsState, is the statistic result of current energy saving state of the cells at a certain time, which can be used by consumers to make analysis (e.g., observed service experience analysis made by NWDAF) as described in TS 28.104 [15] clause 8.4.4.3.

For Redundant Transmission Experience related analytics, NWDAF can leverage the E2E latency analysis output from MDA as defined in TS 28.104 [15] clause 8.4.2.4.2 in additioan to the input as described in TS 23.288 [5] clause 6.13.2.

6.60.1.1	Input data

NWDAF collects following MDAF analytics output from TS 28.104 [15] clause 8.4.2.1.3 and clause 8.4.4.3 for Oberserved Service Experience for a network slice in Table 6.60.1.1-1.

Table 6.60.1.1-1: Data collection from MDAF for Service Experience analytics

| Information | Source | Description |
| --- | --- | --- |
| ServiceExperienceIssueType | MDAF | Indication of the service experience issue type in TS 28.104 [15] 8.4.2.1.3. The allowed value is one of the enumerated values: RAN issue, CN issue, both |
| AffectedObjects | MDAF | The managed object instances where the service experience is applicable, e.g., SubNetwork Instance, NetworkSlice Instance, S-NSSAI in TS 28.104 [15] 8.4.2.1.3. |
| ServiceExperienceStatistics | MDAF | The statistics of the level of service experience for a service in a certain time period, e.g. there are five levels which are represented by 1, 2, 3, 4, 5 where level 1 represents the users are enduring bad experience while level 5 represents the users’ requirements are perfectly satisfied in TS 28.104 [15] 8.4.2.1.3. |
| ServiceExperiencePredictions | MDAF | The predictions of the level of service experience for a service in a certain time period in TS 28.104 [15] 8.4.2.1.3. |
| StatisticsOfCellsEsState | MDAF | The statistic result of current energy saving state of the cells at a certain time in TS 28.104 [15] 8.4.4.3. |

NWDAF collects following MDAF analytics output in TS 28.104 [15] clause 8.4.2.4.3 for Redundant Transmission Experience related analytics in Table 6.60.1.1-2.

Table 6.60.1.1-2: Data collection from MDAF for Redundant Transmission Experience related analytics

| Information | Source | Description |
| --- | --- | --- |
| E2ELatencyIssueType | MDAF | Indication the type of the E2E latency issue. The allowed value is one of the enumerated values: RAN latency issue, CN latency issue |
| AffectedObjects | MDAF | The managed object instances of subnetwork, managed elements or network slices where the latency issue happens |

6.60.2	Procedures

For Observed Service Experience related network data analytics, the procedure in Figure 6.4.5-1 of TS 23.288 [5] is re-used with additional data collection step from MDAF as defined in TS 28.104 [15].

For Redundant Transmission Experience related analytics, the procedure in Figure 6.13.4.1-1 of TS 23.288 [5] is re-used with additional data collection step from MDAF as defined in TS 28.104 [15].

6.60.3	Impacts on Existing Nodes and Functionality

7	Overall Evaluation

Editor's note:	This clause will provide evaluation of different solutions.

8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-02 | SA2#149E | S2-2201066 | - | - | - | Skeleton document proposal | 0.0.0 |
| 2022-03 | SA2#149E |  | - | - | - | Inclusion of documents approved in SA2#149E: S2-2201617, S2-2200821, S2-2201618, S2-2201619, S2-2201620, S2-2201861, S2-2201622, S2-2201632, S2-2201623, S2-2201624, S2-2201625. | 0.1.0 |
| 2022-04 | SA2#150E |  |  |  |  | Inclusion of documents approved in SA2#150E: S2-2203352, S2-2203353, S2-2203354, S2-2203355, S2-2203356, S2-2203357, S2-2203358, S2-2203359, S2-2203360, S2-2203361, S2-2203362, S2-2203363, S2-2202314, S2-2203364, S2-2203365, S2-2203366, S2-2203367, S2-2203368, S2-2203369, S2-2203371, S2-2203372, S2-2202983, S2-2203373, S2-2203374, S2-2203375, S2-2203376, S2-2203377, S2-2203378, S2-2203379. | 0.2.0 |
| 2022-04 | SA2#150E |  |  |  |  | Corrections to editor's note and Figure number | 0.2.1 |
| 2022-05 | SA2#151E |  |  |  |  | Inclusion of documents approved in SA2#151E: S2-2205070, S2-2205071, S2-2205072, S2-2205073, S2-2205074, S2-2205075, S2-2205076, S2-2205077, S2-2205078, S2-2205079, S2-2205080, S2-2205081, S2-2205082, S2-2205083, S2-2205084, S2-2205085, S2-2205086, S2-2205087, S2-2205088, S2-2205089, S2-2204326, S2-2205090, S2-2205091, S2-2205092, S2-2205093, S2-2205094, S2-2205095, S2-2204500, S2-2205096, S2-2205097, S2-2205098, S2-2205099, S2-2205100, S2-2205101, S2-2205102, S2-2205103, S2-2205104, S2-2204084, S2-2205105, S2-2205106, S2-2205107, S2-2205108, S2-2204082, S2-2205109, S2-2205110, S2-2205111, S2-2205112, S2-2205113, S2-2205114, S2-2205115, S2-2205116, S2-2205117, S2-2205118, S2-2205376, S2-2205119 | 0.3.0 |