3GPP TR 23.700-90 V0.3.0 (2021-04)

Technical Report

3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Interconnection and Migration Aspects for Railways; (Release 18)

| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |
| --- | --- |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	8

4	Scenarios	8

5	Key issues	8

5.1	Key issue 1 – Optimize the connectivity between MC systems	8

5.2	Key issue 2 – Functional alias handling	8

5.3	Key issue 3 – Group communication between MC systems	8

5.4	Key issue 4 – Location information with multiple MC systems	9

5.5	Key issue 5 – Quick migration towards another MC system	9

5.6	Key issue 6 – Call forwarding/transfer between MC systems	9

5.7	Key issue 7 – IP connectivity between MC systems	9

6	Architectural requirements	10

7	Solutions	10

7.1	Solution on functional architecture enhancements to support location information	10

7.1.1	General	10

7.1.2	Solution description	10

7.1.2.1	Functional architecture	10

7.1.2.2	Reference points	10

7.1.3	Solution evaluation	10

7.2	Solutions on providing location information	10

7.2.1	General	10

7.2.2	Solution description	11

7.2.2.1	Immediate location information requests and reports	11

7.2.2.2	History location information requests and reports	11

7.2.2.3	Event triggered location information reports based on subscriptions	11

7.2.2.4	Configuration of event triggered location information reports	11

7.2.2.5	Handling of location information subscriptions	11

7.2.3	Solution evaluation	11

7.3	Private call using functional alias towards a partner MC system	12

7.3.1	General	12

7.3.2	Solution description	12

7.3.2.1	Principle	12

7.3.2.2	Configuration	12

7.3.2.3	Messages	12

7.3.2.4	Procedure	13

7.3.3	Solution evaluation	14

7.4	Functional alias support for migrated users	15

7.4.1	General	15

7.4.2	Solution description	15

7.4.2.1	Principle	15

7.4.2.2	Procedure	15

7.4.2.3	Configuration	16

7.4.3	Solution evaluation	16

7.5	Migration during an ongoing group communication	16

7.5.1	General	16

7.5.2	Solution description	17

7.5.2.1	Procedure	17

7.5.3	Solution evaluation	18

7.6	Migration during an ongoing private communication	18

7.6.1	General	18

7.6.2	Solution description	18

7.6.2.1	Procedure	18

7.6.2.2	Information flows	19

7.6.2	Solution evaluation	19

8	Overall evaluation	19

8.1	Key issue and solution evaluation	19

8.1.1	Introduction	19

8.1.2	Results	19

9	Conclusions	20

Annex A (informative): Change history	21

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The work done for interconnection and migration did not fully address the needs for railways, additional work is required. Before starting additional normative stage 2 work on certain aspects, a study is initiated to identify gaps in existing mechanisms on interconnection and on migration and to develop solutions for those gaps. The technical report provides recommendations for solutions which are candidates for normative work.

# 1	Scope

The present document studies solutions to satisfy interconnection and migration needs for railways. It identifies enhancements to be included in the technical specifications for MCPTT (3GPP TS 23.379 [2]), MCVideo (3GPP TS 23.281 [3]), MCData (3GPP TS 23.282 [4]) and in the common functional architecture (3GPP TS 23.280 [5]) to support mission critical communications.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.379:	"Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[3]	3GPP TS 23.281:	"Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[4]	3GPP TS 23.282:	"Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[5]	3GPP TS 23.280:	"Common functional architecture to support mission critical services; Stage 2".

[6]	3GPP TR 23.744: "Study on location enhancements for mission critical services".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Scenarios

Editor's Note:	This clause will provide use cases and scenarios.

# 5	Key issues

## 5.1	Key issue 1 – Optimize the connectivity between MC systems

The operation between multiple MC systems becomes for important and so the path of media and signalling needs to be re-considered. Optimisations on media and signalling routing may save transmission resources and may improve the quality of service.

List of key issues:

-	Study and describe the existing mechanism of routing of media and signalling between MC systems.

-	Investigate optimizations and develop solutions to improve media and signalling routing between MC service systems.

## 5.2	Key issue 2 – Functional alias handling

The use of functional aliases outside the own home MC system has not been considered by current specifications. Same applies for the use of a functional alias between interconnected MC systems, i.e. between a partner MC system and the home MC system as well as between two partner MC systems. Considerations on functional alias management procedures (activation, deactivation or group binding etc.), which are applied outside the own home MC system, are also missing in current specifications.

List of key issues:

-	Describe the related procedures and elaborate the configuration needs for the use of functional alias in a partner MC system which was allocated by the home MC system.

-	Describe the use of functional alias (allocated by the home MC system) used in multiple partner MC systems.

-	Describe functional alias activation and deactivation procedures and configuration needs when the MC service user is migrated into a partner MC system.

-	Elaborate how the transfer and the retrieval of functional alias binding with MC service groups is done when using a functional alias over multiple MC systems.

## 5.3	Key issue 3 – Group communication between MC systems

Group communications require further considerations when MC service users are spread to multiple MC systems or when MC service users re-locate to another MC system during an ongoing group communication.

List of key issues:

-	Develop solutions which enable group communications that are spread to multiple MC systems.

-	Develop solutions which allow migration to another MC system during an ongoing group communication.

-	Elaborate security implications when MC service users are involved that belong to different MC systems.

## 5.4	Key issue 4 – Location information with multiple MC systems

The use of location related procedures when involving users of multiple MC service systems are not covered by current specifications.

List of key issues:

-	Study current location information procedures and configuration data and elaborate its applicability when MC service users operating in multiple MC systems are involved.

-	Develop solutions (incl. required configuration data) to support location information management procedures for operation in multiple MC systems.

-	Elaborate any security related aspects when location information is exchanged and managed between multiple MC systems.

## 5.5	Key issue 5 – Quick migration towards another MC system

For railways there is a need to migrate from one MC system to another MC system (organization) at high speed with no loss of service (regular border crossing scenario).

List of key issues:

-	Study current migration solutions and investigate possible improvements.

-	If required, develop new solutions to support quick migration from one MC system to another MC system.

-	Elaborate which configuration data is required to support quick migration.

-	Elaborate if there are any security implications to support quick migration.

## 5.6	Key issue 6 – Call forwarding/transfer between MC systems

The use of supplementary services between interconnected MC systems, i.e. between a partner MC system and the home MC system as well as between two partner MC systems, has not been specified.

Solutions are needed to enable call forwarding and call transfer procedures between interconnected MC systems.

List of key issues:

-	Develop solutions which enable call forwarding to MC service users in a different MC system.

-	Develop solutions which enable call transfer involving MC service users in a different MC system.

## 5.7	Key issue 7 – IP connectivity between MC systems

IP connectivity related aspects that involves two MC systems needs further investigations. Point-to-point and group standalone transmissions are to be considered as well as scenarios when the MC service user is abroad and uses home breakout or local breakout.

List of key issues:

-	Investigate and develop solutions for IP connectivity point-to-point transmissions that involves two MC systems (support home and local breakout).

-	Investigate and develop solutions for IP connectivity group standalone transmissions that involves two MC systems (support home and local breakout).

# 6	Architectural requirements

Editor's Note:	This clause will provide architectural requirements.

# 7	Solutions

## 7.1	Solution on functional architecture enhancements to support location information

### 7.1.1	General

This solution addresses the key issue 4 described in clause 5.4 on defining a functional architecture when using multiple interconnected MC systems.

### 7.1.2	Solution description

#### 7.1.2.1	Functional architecture

The in 3GPP TR 23.744 [6] clause 6.14 described functional architecture of interconnected MC systems for the exchange of location information is used to support the following scenarios:

a.	immediate location information requests and reports;

b.	history location information requests and reports;

c.	event triggered location information reports based on subscriptions;

d.	configuration of event triggered location information reports; and

e.	handling of location information subscriptions, e.g. subscription requests, cancellation of subscriptions.

#### 7.1.2.2	Reference points

The in 3GPP TR 23.744 [6] clause 6.14 described reference points between entities of interconnected MC systems for the exchange of location information are used to support following scenarios:

a.	utilizing the connection of the MC gateway servers; and

b.	utilizing the connection of the Location management servers.

### 7.1.3	Solution evaluation

The enhancements of the functional architecture described in 3GPP TR 23.744 [6] clause 6.14 fulfil the need for the exchange of location information in case of interconnection.

## 7.2	Solutions on providing location information

### 7.2.1	General

This solution addresses the key issue 4 described in clause 5.4 on defining the usage of the functional architecture when using multiple interconnected MC systems.

Editor's note:	How the Functional Alias(es) is resolved while handling location information of interconnected MC systems is FFS.

### 7.2.2	Solution description

#### 7.2.2.1	Immediate location information requests and reports

Building on 3GPP TR 23.744 [6] clause 6.15.2.7 as well as clause 6.15.2.8 the functional alias(es) may be included to the described information flows.

The on-demand request of location information procedure described in 3GPP TR 23.744 [6] clause 6.15.2.9 is utilized.

#### 7.2.2.2	History location information requests and reports

Building on 3GPP TS 23.280 [5] clauses 10.9.2.11, 10.9.2.12, 10.9.2.13, 10.9.2.14, 10.9.2.15 as well as clause 10.9.1.16 the functional alias(es) may be included to the described information flows.

Building on 3GPP TS 23.280 [5] clauses 10.9.3.9.2.2, 10.9.3.9.3.3 as well as clause 10.9.3.9.4.3 additional procedures are needed to cover cases of the interconnected MC systems.

#### 7.2.2.3	Event triggered location information reports based on subscriptions

Building on 3GPP TR 23.744 [6] clause 6.15.2.4 the functional alias(es) may be included to the described information flow.

The event-triggered location information notification procedure described in 3GPP TR 23.744 [6] clause 6.15.2.6 is utilized.

#### 7.2.2.4	Configuration of event triggered location information reports

Building on 3GPP TR 23.744 [6] clause 6.15.2.14 as well as clause 6.15.2.15 the functional alias(es) may be included to the described information flows.

The location reporting temporary configuration procedure described in 3GPP TR 23.744 [6] clause 6.15.2.16 is utilized.

#### 7.2.2.5	Handling of location information subscriptions

Building on 3GPP TR 23.744 [6] clauses 6.15.2.2, 6.15.2.3, 6.15.2.11 as well as clause 6.15.2.12 the functional alias(es) may be included to the described information flows.

The Location information subscription procedure described in 3GPP TR 23.744 [6] clause 6.15.2.5 is utilized.

The Location information cancel subscription procedure described in 3GPP TR 23.744 [6] clause 6.15.2.13 is utilized.

### 7.2.3	Solution evaluation

The following aspects of the key issue 4 – Location information with multiple MC systems are addressed with the above solution:

-	The current available information flows and procedures in 3GPP TR 23.744 [6] and 3GPP TS 23.280 [5] are analysed for interconnected MC systems. Especially 3GPP TR 23.744 [6] is addressing a wide range of information flows and procedures required for the interconnected MC system use case.

-	Missing procedures for the exchange of history location information are identified and require further investigation for the explicit need with the interconnected MC system use case.

-	The analysed information flows and procedures in 3GPP TR 23.744 [6] and 3GPP TS 23.280 [5] do not show additional security implications for the interconnected MC system use case.

## 7.3	Private call using functional alias towards a partner MC system

### 7.3.1	General

This solution addresses the key issue 2 described in clause 5.2 on functional alias handling.

The solution provides the possibility for an MCPTT user to initiate a private MCPTT call using a functional alias as target address towards an MCPTT user in a partner MCPTT system.

### 7.3.2	Solution description

#### 7.3.2.1	Principle

Allow the MCPTT server in the primary MC system, to send an MCPTT private call request to an MCPTT server in the partner MC system using a functional alias without incorporating the MCPTT ID of the called party.

Extend the MCPTT service configuration data so that the MC server in the primary MC system can identify that the functional alias belongs to a partner MC system. This is done by using certain formats for functional aliases belonging to the partner MC system towards the private call request is to be forwarded.

#### 7.3.2.2	Configuration

The MCPTT service configuration data is extended to contain the list of MC system identities and corresponding functional alias mask (e.g. *.sbb.ch) information for all partner MCPTT systems. Based on the functional alias mask configured for all the partner MCPTT systems, the primary MCPTT server can route the request to the proper partner MCPTT system.

Proposed modifications in 3GPP TS 23.379 [2] Table A.5-2 MCPTT service configuration data (on-network):

Table 7.3.2.2-1: MCPTT service configuration data (on network)

|  | List of partner MCPTT systems where system specific configuration applies for interconnection |  |  |  |
| --- | --- | --- | --- | --- |
|  | > MC system identity of partner MCPTT system | N | Y | Y |
|  | > Functional alias mask | N | Y | Y |

Editor's Note: The use of the same functional alias in multiple MC systems requires further study.

#### 7.3.2.3	Messages

The MCPTT private call request between MCPTT servers is modified to allow only using the functional alias as called party address, i.e. the MCPTT ID address is not resolved and not contained by the primary MCPTT server.

Proposed modifications in 3GPP TS 23.379 [2] Table 10.7.2.1.2-1: MCPTT private call request (MCPTT server to MCPTT server) information elements:

Table 7.3.2.3-1: MCPTT private call request (MCPTT server to MCPTT server) information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCPTT ID (see NOTE) | O | The MCPTT ID of the called party |
| Functional alias (see NOTE) | O | The functional alias of the called party |
| Use floor control indication | M | This element indicates whether floor control will be used for the private call. |
| SDP offer | M | Media parameters of MCPTT client. |
| Requested commencement mode | O | An indication of the commencement mode to be used. |
| Implicit floor request | O | An indication that the user is also requesting the floor. |
| Requested priority | O | Priority level requested for the call. |
| Location information | O | Location of the calling party |
| NOTE: At least one identity must be present. |  |  |

#### 7.3.2.4	Procedure

The MCPTT private call setup procedure between MCPTT servers is modified to allow using the functional alias as called party address, i.e. the MCPTT ID address is not resolved by the primary MCPTT system, instead the primary MCPTT server uses the functional alias mask information configured in the MCPTT service configuration data to send the request towards the proper MCPTT system.

Proposed changes against 3GPP TS 23.379 [2] clause 10.7.2.3.1: Private call setup in automatic commencement mode – MCPTT users in multiple MCPTT systems:

NOTE: The changes are applicable when using manual commencement mode as well.

Figure 7.3.2.4-1: Private call setup in automatic commencement mode – users in multiple MCPTT systems

1./2.	No change.

3.	The MCPTT private call request contains the MCPTT ID or functional alias of invited user.

Editor's Note:	Further study is needed when the target address is a functional alias and end-to-end encryption is used for this call.

4./5.	No change.

6.	If the MCPTT private call request is initiated by using a functional alias as called party address, then the target MCPTT system is identified by help of configured functional alias mask in the MCPTT service configuration data.

7.	If the MCPTT server private call request contains a functional alias as called party address, the MCPTT server resolves the MCPTT ID of the called MCPTT user by using the functional alias address.

8.-12.	No change.

### 7.3.3	Solution evaluation

Editor's Note:	This clause will provide the evaluation of this solution.

## 7.4	Functional alias support for migrated users

### 7.4.1	General

This solution addresses the key issue 2 described in clause 5.2 on functional alias handling.

The solution provides the possibility to configure proper functional aliases for MC service users migrating to a partner MC system.

### 7.4.2	Solution description

#### 7.4.2.1	Principle

The solution relies on the existing procedure "MC service user receiving MC service from a partner MC system" described in 3GPP TS 23.280 [5] clause 10.1.4.3.2. The procedure can provide the list of functional aliases for use in the partner MC system as part of the migration MC service user profile.

#### 7.4.2.2	Procedure

A set of functional aliases for each primary MC system is configured in the partner MC system. After the MC user is migrated from the primary MC system into the partner MC system, the partner MC system provides the MC user profile for migration which includes a preconfigured set of functional aliases allowed to be used within the partner MC system.

The MC configuration management server provides the functional alias management server functionality as described in subclause 7.4.2.2.13 of 3GPP TS 23.280 [5]. The configuration management server functionality is extended with the configuration function for functional alias sets for different partner MC system.

NOTE:	Different migrated MC users from different partner MC systems may need different functional alias sets; i.e. the configuration acts on MC partner system basis.

The procedure is described in 3GPP TS 23.280 [5] clause 10.1.4.3.2: MC service user receiving MC service from a partner MC system:

Figure 7.4.2.2-1: Retrieval of user profile in partner MC system

Within step 5 above the partner MC system has the option of modifying the MC user profile for migrated users. Step 5 is enhanced to support functional alias sets for use in the partner MC system.

After the MC service UE has received the MC service user profile in step 9, the MC service user is able to activate functional aliases within the partner MC system and use them for communication in the partner MC system.

Editor's Note: The use of the same functional alias in both, the primary and the partner MC system, requires further study.

#### 7.4.2.3	Configuration

Editor's Note: This clause will provide the configuration data needed for this solution.

### 7.4.3	Solution evaluation

Editor's Note:	This clause will provide the evaluation of this solution.

## 7.5	Migration during an ongoing group communication

### 7.5.1	General

This solution addresses the key issue 3 described in clause 5.3 on group communication between MC systems.

The solution provides the capability for an MC service user to migrate to another MC system during an ongoing group communication and to continue the group communication in the other MC system.

### 7.5.2	Solution description

#### 7.5.2.1	Procedure

The procedure is based on the following existing procedures:

-	MC service group de-affiliation procedure as described in TS 23.280 [5] clause 10.8.4.2, or

-	De-affiliation from MC service group(s) defined in partner MC service system and is described in TS 23.280 [5] clause 10.8.4.3.

-	MC service user receiving MC service from a partner MC system as described in TS 23.280 [5] clause 10.1.4.3.2.

-	MC service group affiliation procedure as described in TS 23.280 [5] clause 10.8.3.1, or

-	Affiliation to MC service group(s) defined in partner MC system as described in TS 23.280 [5] clause 10.8.3.2 or clause 10.8.3.2a.

-	Late entry pre-arranged group call as described in TS 23.379 [2] clause 10.6.2.3.1.1.4.

NOTE 1: The solution is about MCPTT group calls but is applicable for other services too.

Pre-conditions:

1.	The MCPTT client is a receiving party in one or more ongoing group calls in the primary MC system.

2.	The MCPTT UE detects the need to change the MC system.

Figure 7.5.2.1-1:	Migration to partner MC system during an ongoing group calls

1.	The MCPTT client requests de-affiliation from MCPTT groups. The MCPTT groups are either defined in the primary MC system (TS 23.280 [5] clause 10.8.4.2) or the partner MC system (TS 23.280 [5] clause 10.8.4.3). The MCPTT client de-authorizes from the primary MC System.

2.	After migration to the partner MC system, the configuration management client triggers retrieval of the MC service user profile used within the partner MC system (TS 23.280 [5] clause 10.1.4.3.2).

NOTE 2:	User authentication, service authorisation and signalling plane procedures are not shown.

3.	The MCPTT client requests affiliation to MCPTT groups. The MCPTT groups are either defined in the primary MC system (TS 23.280 [5] clause 10.8.3.1) or the partner MC system (TS 23.280 [5] clause 10.8.3.2 or clause 10.8.3.2a).

4.	If any of the received group calls are ongoing in the partner MC system, the partner MC system shall initiate a late-entry procedure towards the MCPTT client. If any of the received group calls are taken place in the primary MC system but not yet in the partner MC system, the affiliation by the migrated MCPTT UE triggers the late-entry procedure which then includes the MCPTT UE and the partner MC system into the group call.

The MCPTT client may indicate the successful migration of group communications to the MCPTT user.

### 7.5.3	Solution evaluation

Editor's Note:	This clause will provide the evaluation of this solution.

## 7.6	Migration during an ongoing private communication

### 7.6.1	General

This solution addresses the key issue 5 described in clause 5.5 on quick migration towards another MC system.

The solution provides the capability for an MC service user to migrate to another MC system during an ongoing private communication and to continue the private communication in the other MC system without MC service user interaction.

### 7.6.2	Solution description

#### 7.6.2.1	Procedure

NOTE 1:	The solution is about MCPTT private calls but is applicable for other services too.

Pre-conditions:

1.	The MCPTT client has one or more ongoing private calls in the primary MC system.

2.	The MCPTT UE detects the need to change the MC system.

Figure 7.6.2.1-1:	Migration to partner MC system during an ongoing private call

1.	MCPTT client 1 requests private call suspend to put the call into suspended state in MCPTT client 2. The call is cleared between MCPTT server 1 and MCPTT server 2, and knowledge of the suspended call is held by MCPTT client 1 and MCPTT client 2. The MCPTT users of MCPTT client 1 and MCPTT client 2 get an indication that the private call has been suspended. MCPTT client 1 and MCPTT client 2 start a timer to allow the call suspended state to be cleared if the call is not resumed within a predetermined time interval.

2.	After migration to the other MC system, the configuration management client 1 triggers retrieval of the MC service user profile used within the partner MC system (TS 23.280 [5] clause 10.1.4.3.2).

NOTE 2:	User authentication, service authorisation and signalling plane procedures are not shown.

3.	MCPTT client 1 requests a new private call to MCPTT client 2, with the call resume indication to remove the call suspended state in MCPTT client 2. The MCPTT users of MCPTT client 1 and MCPTT client 2 get an indication that the private call has been resumed.

NOTE 3:	Any local call restrictions are considered.

NOTE 4:	If another private call request is sent to MCPTT client 1 or MCPTT client 2 before the call has resumed, the actions of the receiving MCPTT client are outside the scope of the present document, and could include rejecting this new private call request.

The MCPTT client may indicate the successful migration of private call communications to the MCPTT user.

#### 7.6.2.2	Information flows

Editor's Note:	This clause will provide the detailed information flows of this solution.

### 7.6.2	Solution evaluation

Editor's Note:	This clause will provide the evaluation of this solution.

# 8	Overall evaluation

## 8.1	Key issue and solution evaluation

### 8.1.1	Introduction

All the key issues and solutions specified in this technical report are listed in table 8.1.2-1. It includes the mapping of the key issues (clause 5) to the solutions (clause 7) and corresponding solution evaluations.

In addition, table 8.1.2-1 lists the impacts to other working groups that will need consideration during the Rel-18 normative phase.

### 8.1.2	Results

Table 8.1.2-1: Key issues, solutions, and solution evaluations

| Key issues | Solution | Evaluation (clause reference) | Dependency on other working groups |
| --- | --- | --- | --- |
| Key issue 1 – Optimize the connectivity between MC systems |  |  |  |
| Key issue 2 – Functional alias handling | Clause 7.3 Private call using functional alias towards a partner MC system |  |  |
|  | Clause 7.4 Functional alias support for migrated users |  |  |
| Key issue 3 – Group communication between MC systems |  |  |  |
| Key issue 4 – Location information with multiple MC systems | Clause 7.1 Functional architecture |  |  |
|  | Clause 7.2 Location information |  |  |
| Key issue 5 – Quick migration towards another MC system |  |  |  |
| Key issue 6 – Call forwarding/transfer between MC systems |  |  |  |
| Key issue 7 – IP connectivity between MC systems |  |  |  |

# 9	Conclusions

Editor's Note:	This clause is intended to list conclusions that have been agreed during the study item activities.

# Annex A (informative): Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-08 | SA6#39-e |  |  |  |  | S6-201345 (TR skeleton) | 0.0.0 |
| 2020-09 | SA6#39-e |  |  |  |  | S6-201346, S6-201347, S6-201351, S6-201352, S6-201353, S6-201355, S6-201356, S6-201357, S6-201565 | 0.1.0 |
| 2021-01 | SA6#41-e |  |  |  |  | S6-210092, S6-210093, S6-210204, S6-210205 | 0.2.0 |
| 2021-04 | SA6#42-bis-e |  |  |  |  | S6-210772, S6-210776, S6-210794, S6-210957, S6-210958, S6-210972 | 0.3.0 |