| 3GPP TR 23.700-96 V0.6.0 (2022-05) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on 5G-enabled fused location service capability exposure; (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	8

3.1	Definitions	8

3.2	Abbreviations	8

4	Architectural assumptions and requirements	9

4.1	Architectural assumptions	9

4.2	Architectural requirements	9

5	Key issues	9

5.1	Key issue #1: Architecture enhancement of application enablement for location	9

5.2	Key issue #2: Support of LCS QoS	10

5.3	Key issue #3: Location service differentiation	10

5.4	Key issue #4: Support for hybrid positioning and location fusion	10

5.5	Key issue #5: Initialization and configuration for fused location service	11

6	Functional architecture	12

7	Solutions	12

7.1	Solution #1: Standalone functional architecture for fused location service	12

7.1.1	Solution description	12

7.1.1.1	Functional architecture	12

7.1.1.2	Functional components and reference points	13

7.1.1.3	Merged Architecture to support interaction between FLS and SEAL LMS	14

7.1.2	Solution evaluation	15

7.2	Solution #2: Support of both LCS and SUPL at Fused Location Server	15

7.2.1	Solution description	15

7.2.1.1	Architectural models	15

7.2.2	Solution evaluation	16

7.3	Solution #3: Functional architecture for fused location service leveraging SEAL location management	16

7.3.1	Solution description	16

7.3.1.1	Functional architecture	16

7.3.1.2	Functional components and reference points	17

7.3.1.3	Deployment models	17

7.3.2	Solution evaluation	18

7.4	Solution #4: Location service registration	18

7.4.1	Solution description	18

7.4.2	Solution evaluation	19

7.5	Solution #5: Location profiling for supporting fused location service enablement	19

7.5.1	Solution description	19

7.5.1.1	General	19

7.5.1.2	Procedure	20

7.5.2	Solution evaluation	22

7.6	Solution #6: Location service configuration	22

7.6.1	Solution description	22

7.6.1.1	Service flow for fused location service configuration	22

7.6.2	Solution evaluation	23

7.7	Solution #7: Location QoS based location sources and positioning methods selection	23

7.7.1	Solution description	23

7.7.1.1	Procedure of location QoS based location sources and positioning methods selection	25

7.7.2	Solution evaluation	25

8	Overall evaluation	25

9	Conclusions	26

9.1	General conclusions	26

9.2	Conclusions of key issue #x	26

Annex A: Change history	27

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In drafting the TS/TR, pay particular attention to the use of modal auxiliary verbs! TRs shall not contain any normative provisions.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document studies and evaluates the application architecture aspects and solutions to address potential new and enhanced location capabilities for vertical application enabler, including the following aspects:

-	Enabling location performance (accuracy, availability and latency) enhancements through combined use and fusion of 3GPP and non-3GPP location technologies at the application layer;

-	Enabling continuity of location services in different environments at the application layer;

-	Identification and configuration of location related requirements (including location QoS) for vertical application services with the use of SA2-defined mechanisms;

-	Architecture enhancement leveraging 5G positioning and location services;

-	Location data handling capabilities (e.g. logging, management);

-	Enhanced efficiency on location service processing (e.g. by leveraging edge computing capabilities);

-	Enabling value-added location service capabilities exposure to vertical applications;

-	Enhancements on SEAL location management addressing the aspects above.

NOTE:	This study will not duplicate solutions already available in Core Network and RAN.

Editor's Note: The aspects above need to be aligned with the conclusions of the study.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

[3]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[4]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[5]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[6]	3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[7]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains; Stage 1".

[8]	3GPP TS 22.125: "Unmanned Aerial System (UAS) support in 3GPP; Stage 1".

[9]	3GPP TS 22.071: "Location Services (LCS); Service description; Stage 1".

[10]	Open Mobile Alliance, OMA AD SUPL: "Secure User Plane Location Architecture", (http://www.openmobilealliance.org).

[11]	Open Mobile Alliance, OMA AD MLS: "Mobile Location Service Architecture", (http://www.openmobilealliance.org).

[12]	3GPP TS 23.502: " Procedures for the 5G System (5GS); Stage 2".

[13]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[14]	3GPP TS 29.572: "Location Management Services; Stage 3".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

For the purposes of the present document, the following terms and definitions given in TS 22.261 [2] apply:

5G enhanced positioning area: see TS 22.261 [2].

5G positioning service area: see TS 22.261 [2].

For the purposes of the present document, the following terms and definitions given in TS 23.271 [3] apply:

Current Location: see TS 23.271 [3].

LCS Server: see TS 23.271 [3].

Location Estimate: see TS 23.271 [3].

Velocity: see TS 23.271 [3].

For the purposes of the present document, the following terms and definitions given in TS 23.273 [4] apply:

LCS Client: see TS 23.273 [4].

For the purposes of the present document, the following terms and definitions given in TS 23.501 [5] apply:

5G Access Network: see TS 23.501 [5].

5G Core Network: see TS 23.501 [5].

5G System: see TS 23.501 [5].

NG-RAN: see TS 23.501 [5].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

GMLC	Gateway Mobile Location Centre

LCS	LoCation Services

LDR	Location Deferred Request

LMF	Location Management Function

LPP	LTE Positioning Protocol

MO-LR	Mobile Originated Location Request

MT-LR	Mobile Terminated Location Request

NI-LR	Network Induced Location Request

SLP	SUPL Location Platform

SUPL	Secure User Plane Location

TNAN	Trusted Non-3GPP Access Network

UNAN	Untrusted Non-3GPP Access Network

# 4	Architectural assumptions and requirements

## 4.1	Architectural assumptions

For discussions of architecture aspects of application enablement in this document, the following architectural assumptions apply:

-	The positioning methods include the standard positioning methods supported for NG-RAN access as specified in clause 4.3 in 3GPP TS 38.305 [6], and the positioning methods supported for non-3GPP access as specified in clause 5.3.1 in 3GPP TS 23.273 [4].

-	The source of location may include other standard location service (non-3GPP location service of e.g. OMA) from the third party.

-	The PLMN operator must be able to protect user location data and privacy when the application enablement architecture is within the PLMN operator domain.

## 4.2	Architectural requirements

Editor's Note:	This clause will describe the architectural requirements.

# 5	Key issues

## 5.1	Key issue #1: Architecture enhancement of application enablement for location

The 3GPP TS 22.261 [2], 3GPP TS 22.104 [7], 3GPP TS 22.125 [8] have specified high accuracy positioning requirements of 5G for the support of various vertical applications. The major aspects of the study are the support of fusion or combination of different location technologies and the value added location capabilities at the application enabler, so that to enable the enhanced performance and meeting the vertical's needs.

It should be studied whether and how the SEAL location-related architecture enhancement or new architecture model is needed based on evaluations of existing location management architectures, functional entities and capabilities with following aspects:

-	Functional entit(ies) supporting the combined use and fusion of different location technologies at the application layer;

-	Architecture enhancement addressing the consideration of flexibility, scalability and reliability;

-	Support for high-accuracy positioning based on LCS and 5G network exposure;

-	Providing value-added location servicies to accommodate new service requirements and the evolving application enablement capabilities;

-	Architecture aspects that make sure UE provided location is not spoofed;

-	Architecture aspects that comply with local, national, and regional location privacy requirements;

-	Architecture aspects exploiting edge computing capabilities including EDGEAPP and 5GC edge capabilities.

The MNO may deploy both LCS and SUPL due to deployment and cost considerations while achieving consistent location performances. It is for further study:

-	The possible application layer architectural models to support both SUPL and LCS.

-	The potential scenarios that LCS and SUPL location towards the same target UE are received by application layer at the same time.

-	The architectural aspects to support selection or coordination between LCS and SUPL at the application layer when both location services are supported.

## 5.2	Key issue #2: Support of LCS QoS

According to 3GPP TS 23.273 [4] and 3GPP TS 22.071 [9], the LCS QoS which is characterised by LCS QoS Class, Accuracy and Response Time may be required by the application (LCS client) for location requests. For certain LCS services the LCS QoS Class is non-negotiable.

To support the LCS QoS the following aspects need to be studied:

-	How to support invocation of LCS service (as defined by SA2) with a required LCS QoS, including how and when the LCS QoS attributes are specified in an application scenario, and how to potentially use the LCS QoS attributes differently for different vertical scenarios;

-	How to support the identification of an appropriate LCS QoS requirement between all interested parties.;

-	How to potentially retrieve the continuity and consistency of LCS QoS for the vertical applications;

-	How to potentially support the negotiation of required LCS QoS that is application driven.

## 5.3	Key issue #3: Location service differentiation

Within the core network the LCS client has been categorized (as LCS client type) such that the privacy check, potitioning methods schemes and other procedures can be differentiated.

Within the application enabler layer there are also the dimensions to distinguish location services so that the service handling (e.g. priority information, service coverage, geographical area and other information used to invoke LCS service, the selection of location source, etc.) is differentiated. Also the location services distinguished based on use cases and regulation need to be considered.

It is for further study:

-	The possible dimensions to distinguish location service in application enabler layer and how to enable the location service differentiation.

## 5.4	Key issue #4: Support for hybrid positioning and location fusion

The location related KPIs of the various use cases, verticals, scenarios and application services (e.g. V2X, IIoT, UAS) not only require varying levels of accuracy (horizontal, vertical, 2D, 3D), but also different location fix latency requirements, availability requirements, speed and heading, etc.

Depending on the use case (which may be driven by the vertical/ASP needs) different location services may be used to provide hybrid position estimates and the coordination may be needed to combine or fuse location from multiple sources.

The application enabler layer could support possible coordination for location fusion, collecting and processing the location reports from different location services on behalf of the vertical/ASP. Such support capability will require enhancements at the enabler layer and potentially new APIs.

This key issue aims to study:

-	Whether and how the enabler layer could provide support for hybrid positioning and location fusion.

-	Whether and how the enabler layer could support efficiently translating a vertical/ASP requirement to the processing for location collection, combining and fusion based on factors e.g. the application type, environment, capabilities, etc.

## 5.5	Key issue #5: Initialization and configuration for fused location service

The application enabler needs to consider how the location service based on multiple or various location sources is initiated and configured.

This key issue aims to study:

-	How to initiate and start the fused location service for a target UE in different scenarios, environment, network condition, type of service and etc.

-	How to initiate and start the fused location service such that the location capabilities of target UE and the application layer location service can be coordinated.

-	What configurations are needed for the initialization of fused location service and how.

-	What application layer sessions are established for the fused location service and how.

## 5.6	Key issue #6: Value-Added Location Services

How does the FLS/SEAL LMS get the UE location from 3GPP and non-3GPP defined accesses are described in this TR, TS23.273 and TS23.434. A lot of value-added location services provide location enhanced functions to the upper layer consumer. Mobile internet and industrial application can use one or more value-added location services. Different value-added location services may require different architectures, interfaces, functions and procedures.

The following aspects can be addressed in the study:

-	Identify which value-added service is to be studied and provide the functional description?

The following value-added location services can be studied, and more value-added services can be further included and studied in the solution part (not exhausted):

-	Location format mapping

-	Location Event Trigger provision, invoke, revoke

-	Periodic and or event Triggered location reporting

-	Real time location information Pushing

-	Geofencing

-	(Indoor) Map provision

-	Location Alerting

-	Real time Tracing request or playback (continuous locations in a map)

-	History Trace request or playback

-	Time information of the first entering and the last leaving an area (e.g. working campus)

-	The length of time to stay in an area

-	The times to re-enter and re-leave an area

-	Location information analysis

-	Heatmap

-	Speed

-	Heading Direction

-	Identify whether the value-added service as listed above can be supported based on the procedures defined for the SEAL LMS and FLS?

NOTE:	Existing location service may be enhanced if not fully supported.

# 6	Functional architecture

Editor's Note:	This clause will describe the functional model.

# 7	Solutions

## 7.1	Solution #1: Standalone functional architecture for fused location service

### 7.1.1	Solution description

This solution addresses key issue #1: Architecture enhancement of application enablement for location.

#### 7.1.1.1	Functional architecture

The figure 7.1.1.1-1 identifies the architecture of fused location service enabled by 5GS.

Figure 7.1.1.1-1: 	Functional architecture of fused location service

The architecture is composed of logical function modules that are not necessarily physical entities and can reside in or co-locate with existing application layer entities as appropriate.

In the architecture, the Fused Location Server (FLS) and Application Specific Server can be within the MNO domain or third party service provider domain.

The FLS architecture supports multiple possible sources of location information including:

-	AMF location service exposed by NEF (as defined in 3GPP TS 23.502 [12]);

-	LCS location retrieved from either NEF or GMLC (as defined in 3GPP TS 23.273 [4]);

-	SUPL location retrieved from SLP (as defined in OMA AD SUPL [10]);

NOTE:	Whether and how the SUPL location service can be exposed by 5GC is within the remit of SA2.

-	Certain RAT-independent positioning retrieved from 3rd party location server or Fused Location Client.

-	Retrieve the target UE Positioning via the FLS-1 interface.

Editor's Note: It is FFS to show the relationship of Fused Location Function with SEAL-LM.

#### 7.1.1.2	Functional components and reference points

The Fused Location Server provides location information of the target UE based on positioning or location data retrieved from one or multiple location sources. The Fused Location Server can get the location information via the non-3GPP defined access from the Fused Location Client and additionally can get location information via the 3GPP access. The Fused Location Server selects one or more access types, one or more location methods, and Control Plane /User Plane (e.g. SUPL [10]) methods based on the requested location QoS. The Fused Location Server provides a normalized description of location data to the application-specific server (e.g. of ecosystem partners) through the northbound API.

For the non-3GPP defined accesses, the FLS also needs to support the five types of interaction with the Fused Location Client to retrieve the location information via the non-3GPP defined accesses:

-	Get the UE location information;

-	Provide location notification to the FLC in the target UE and get a guarantee to get location information from the target UE;

-	Install location event triggers in the FLC in the target UE to support the target UE terminated deferred location information;

The 3rd Party Location Server provides the location of a certain location technology (typically the network-based positioning) e.g. Bluetooth.

The Fused Location Client represents the client of the target UE providing the UE-based positioning via the non-3GPP defined access and location-related information and provides the UE location information via the non-3GPP defined access to the Fused Location Server via an IP connection.

NOTE: 	How does the Fused Location Client get the UE location information via the non-3GPP defined access is out of scope of 3GPP.

The NEF (as defined in 3GPP TS 23.501 [5]) exposes location service of 5GC when Fused Location Server is external to MNO domain.

The GMLC (as defined in 3GPP TS 23.273 [4]) provides LCS when Fused Location Server is within the MNO domain.

The SLP (as defined in OMA AD SUPL [10]) provides location of SUPL network.

The interfaces are described as followed:

FLS-1:	Reference point supporting location reporting, location determination, location management and exchange of location contextual information (e.g. UE ID, location capabilities of the non-3GPP defined access, the available non-3GPP defined accesses ) over application layer transactions between the Fused Location Server and the Fused Location Client of the target UE. The FLS-1 may support HTTP or WebSocket and the IP connection between the Fused Location Client and Fused Location Server

NOTE:	The IP connection between the Fused Location Client and Fused Location Server can be provided by the 5GS PDU Session.

FLS-2:	Service-based interface exposing fused location data   to the applications (e.g. the vertical applications, the applications of ecosystem partners, etc.). The FLS-2 may support HTTP or WebSocket.

FLS-3:	Reference point between the FLS and a database for storing and retrieving location information for the target UE and user profile for the target UE.

NOTE:	The definition of FLS-3 is out of scope of this specification.

FLS-4:	The reference point is used for location retrieval of the target UE from that 3rd party location server. The FLS-X can be a service-based interface. The FLS-4 may support HTTP or WebSocket.

NOTE:	The definition of FLS-4 is out of scope of this specification.

Nnef:	Service-based interface as defined in 3GPP TS 23.501 [5].

Le:	Reference point as defined in OMA AD MLS [11].

LM-UU:	Reference point as defined in 3GPP TS 23.434 [13].

LM-S:		Reference point as defined in 3GPP TS 23.434 [13].

NOTE:	If the UE supports MUSIM, the FLS can get the UE location information from the PLMNs via the LM-S reference of each PLMN.

#### 7.1.1.3	Merged Architecture to support interaction between FLS and SEAL LMS

FLS fuses different location information from multiple resources and provides a better location service/information to the Application Server via its northbound API. And the SEAL LMS can be one of its location sources as described in figure 7.1.1.3-1

The SEAL LMS does not support getting location information from the non-3GPP defined access, the FLS needs to have the interface FLS-1 to get location information from the non-3GPP defined access.

The FLS needs to get the location information from other PLMNs if the target UE is with multiple PLMN accesses, in such cased, the FLS-1 and or the LM-S reference point is to provide such location information from different PLMNs.

The FLS-3 reference point is defined for storing and retrieving location information for the target UE and user profile for the target UE.

Figure 7.1.1.3: Merged architecture with Interaction between Fused Location Server and SEAL LMS

### 7.1.2	Solution evaluation

The merged architecture defined in figure 7.1.1.3-1 is the architecture to merge the Fused Location Server.

Based on the merged architecture, the SEAL LMS needs to upgrade to support Le interface.

SEAL LMS only gets the location information for the target UE via the 3GPP defined accesses and provides the location information to the FLS via the LM-S interface, additionally, the FLS gets the location information via the non-3GPP defined accesses from the FLS-1 reference point.

## 7.2	Solution #2: Support of both LCS and SUPL at Fused Location Server

### 7.2.1	Solution description

This solution addresses key issue #1 in respect of application layer support of both LCS and SUPL.

#### 7.2.1.1	Architectural models

According to OMA AD SUPL [10], Secure User Plane Location (SUPL) is an Enabler which utilizes existing standards where available and possible, to transfer assistance data and positioning data over a User Plane bearer, such as IP, to aid network and SUPL Enabled Terminal (SET) based positioning technologies in the calculation of a SET's position.

When MNO deploys both LCS and SUPL, the 3GPP TS 23.271 [3] defines the operation of SUPL in EPC, in which the location of SUPL can be retrived from GMLC. The potential interworking of control plane LCS and OMA SUPL both deployed by an MNO is outside the scope of that specification. For 5G LCS the support for SUPL is not defined in 3GPP TS 23.273 [4]. The 3GPP TS 38.305 [6] Annex A (informative) provides an architecture for interworking between LCS and OMA SUPL, in which the location of SUPL is directly exposed from SLP without the involvement of core network entities.

Consiering the existing specifications regarding architecture aspects of SUPL, the potential architectural models of application layer consisting the location source of LCS and SUPL are shown in figure below.

Figure 7.2.1.1-1: Possible Fused Location Server architectural models consisting location source of LCS and SUPL

The potential architectural models includes:

a)	LCS and SUPL locations are exposed by 5GC (from GMLC) through the same interface (Le) to the LCS client which is within the MNO domain.

b)	LCS and SUPL locations are exposed by 5GC (from GMLC) and OMA SLP separately, the LCS client and SUPL agent are within the MNO domain.

c)	LCS and SUPL locations are exposed by 5GC (from NEF) and OMA SLP separately to the AF and SUPL agent that are external to the MNO domain.

### 7.2.2	Solution evaluation

This solution is based on the architecture in solution #1 and defines how to fuse the location information from different sources via different interfaces and the possible architecture models.

Editor's note: Updates to solution evaluation based on Solution #1 is FFS.

## 7.3	Solution #3: Functional architecture for fused location service leveraging SEAL location management

### 7.3.1	Solution description

This solution leverages SEAL to address key issue #1: Architecture enhancement of application enablement for location.

#### 7.3.1.1	Functional architecture

The figure 7.3.1.1-1 identifies the architecture of fused location service enabled by 5GS leveraging SEAL.

Figure 7.3.1.1-1:	 Functional architecture of fused location service leveraging SEAL location management

Editor's note:	Representing LMC offering service (e.g. clause 9.3.4 in 23.434) to LMS is FFS.

Editor's note: How to prevent the loop communication between the FLF and LMS when Application Specific Server is consuming their services is FFS

In the architecture, the SEAL Location management server (LMS) interacts with the Fused location function for fusion of different location technologies.

The architecture supports multiple sources of location information including:

-	Location of AMF service exposed by NEF (as defined in 3GPP TS 23.502 [12]);

-	LCS location retrieved from either NEF or GMLC (as defined in 3GPP TS 23.273 [4]);

-	SUPL location retrieved from SLP (as defined in OMA AD SUPL [10]);

-	Location retrieved from 3rd party location server or location management client.

#### 7.3.1.2	Functional components and reference points

The Location management server provides location information of the target UE based on positioning or location data retrived from one or multiple location sources.

Fused location function provides normalized description of location data to the Location management server through the API.

The 3rd Party Location Server provides the location of a certain location technology (typically the network-based positioning) e.g. Bluetooth.

The Location management client represents the client of the requestor/sender UE for location reporting as defined in 3GPP TS 23.434 [13].

The Fused location client represents the client of the sender UE for location reporting via non-3GPP access over FLS-X1 interface.

The location management server is a functional entity that receives and stores user location information and provides user location information to the vertical application server as defined in 3GPP TS 23.434 [13]. The location management server acquires location information from one or more sources including:

-	the NEF (as defined in 3GPP TS 23.501 [5] and 3GPP TS 23.273 [4]) via N33 reference point;

-	the GMLC (as defined in 3GPP TS 23.273 [4]) via Le reference point;

-	the SLP (as defined in OMA AD SUPL [10]) of SUPL network;

-	the Fused location function; and

-	the location management client.

The VAL client and VAL server (vertical application layer entities) are as defined in 3GPP TS 23.434 [13].

The interfaces are described as followed:

LM-UU:	Reference point as defined in 3GPP TS 23.434 [13].

LM-S:		Reference point as defined in 3GPP TS 23.434 [13].

FLS-X2:	The reference point is used for location retrieval of the target UE from 3rd party location server.

NOTE:	The definition of FLS-X2 is out of scope of this specification.

N33:	Service-based interface as defined in 3GPP TS 23.501 [5].

Le:	Reference point as defined in OMA AD MLS [11].

FLS-S:	Reference point used for fused location retrieval from Fused location function either by VAL Server or by Location management server.

Editor's note:	Impacts of architectural changes to existing SEAL LM procedures in 3GPP TS 23.434 [13] are FFS.

Editor's note: The enhancements to location management client, LM-UU and LM-S providing fused location service are FFS.

#### 7.3.1.3	Deployment models

In deployments where Fused Location Function resides outside of SEAL LMS is shown by Figure 7.3.1.1-1. In another deployment the Fused Location Function can be collocated within SEAL LMS. Such function model is as shown in Figure 7.3.1.3-1.

NOTE:	This deployment option could be considered as the functional architecture diagram for 5GFLS during normative specification.

Figure 7.3.1.3-1:	 Functional architecture of fused location service collacted within SEAL location management

Editor's note: Moving deployment model to another clause is FFS

### 7.3.2	Solution evaluation

Editor's Note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

## 7.4	Solution #4: Location service registration

### 7.4.1	Solution description

This solution addresses key issue #5: Initialization and configuration for fused location service.

The procedure for location service registration is illustrated in figure below. This procedure is based on fused location architecture of solution#1. The purpose of this procedure is for the fused location client to register to the location services available at the fused location server while ensuring the privacy of the user.

If the Multi-USIM is supported by the fused location client, the fused location client performs the registration procedure for each PLMN of the Multi-USIM PLMNs.

Figure 7.4.1-1: Location service registration procedure

1.	The fused location client of a target UE sends location service registration request to the fused location server via the FLS-1 reference point, carrying the identifier of the UE (e.g. GPSI,  UUID, etc.) and non-3GPP defined access location capabilites (e.g. the available non-3GPP defined access types, the location methods, the location accuracy and latency of the non-3GPP defined access types).

To access the FLS via the FLS-1 reference point, the fused location client can use any available non-3GPP defined accesses to send the location service registration request.

The fused location server can be pre-configured in the UE or is discovered via the DNS query.

2.	The fused location server checks authorization for the UE's request. If the fused location server supports privacy checking it also performs or assists with e.g. 5GC on privacy check.

3.	The fused location server, upon successful authorization and privacy check (if any), responds to the fused location client with registration result and stores the UE idnetifier information and non-3GPP defined access location capabilities into the UE location context.

If the Multi-USIM is supported by the fused location client, the fused location client performs the steps 1 to 3 to register its identifier (e.g. MSISDN) associated with each PLMN of the Multi-USIM PLMNs .

### 7.4.2	Solution evaluation

This solution is based on the architecture proposed in Solution #1. the fused location client can provide its UE IDs and the location capabilities of the available non-3GPP defined accesses to the FLS, and the FLS can generate and store these inforation as the UE location context. The FLS can use these UE location context in the following location procedures.

## 7.5	Solution #5: Location profiling for supporting fused location service enablement

### 7.5.1	Solution description

#### 7.5.1.1	General

The solution discusses the creation of Location profiles at a fused location service at application enablement layer; and the mapping of Location profiles to one or more vertical applications. Such profiling is based on a variety of factors which correspond to use-case / vertical driven hybrid positioning requirements and policies. This notion of profiling (which is different from the service profile) is based on the various factors which may affect the positioning methods and QoS, and fixes also the communication parameters per vertical requirement-use case, without providing any additional information at the 3rd party app. The attributes that can be used for the Location profile can be some the following (however how the Location profiles are constructed can be up to implementation):

-	the vertical / application service type;

-	the environment (indoor/outdoor, urban/suburban,);

-	the QoS requirements, e.g. accuracy;

-	the capabilities of the UEs involved;

-	the energy constraints for the devices;

-	preference on certain positioning methods, e.g. RAT-dependent or RAT-independent methods;

-	LCS service level (in case of IIOT vertical);

-	priorities of location methods;

-	whether location augmentation is required;

-	whether location prediction is required;

-	whether sidelink positioning assistance is required;

-	whether proximity-based location estimate is required;

-	whether location verification is required.

An example Location profile can be shown in Table 7.5.1.1-1 below.

Table 7.5.1.1-1: Exemplary location profile attributes

| Profile ID / name | Vertical / use case, environment | Positioning Service Level (for IIOT) / QoS / accuracy | Positioning Method(s) / Priorities | Involved 3GPP functionalities / Priorities | Involved non-3gpp access networks | Required APIs / API info | Other |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Location profile #1 | Industrial scenario, indoors, mobile robots/ AGVs | Service Level 6 / cm level accuracy / absolute/relative/ both | 1. DL-TDOA, 2. UL-TDOA, 3. Multi-RTT methods, 4. WLAN, 5. motion sensors, 6. bluetooth | 1. LMF, 2. RAN-LMC, 3. SEAL LMS, .. | 4. WLAN ID,... | NEF APIs, SEAL APIs, .. | Verification / augmentation required |
| Location profile #2 | V2X, outdoor, .. | Decimeter level accuracy /... absolute/relative/both | 1. DL-TDOA, 2. Multi-RTT methods, 3. GNSS-RTK, 4. Sensor fusion, 5. A-GPS | 1. LMF, 2. SEAL LMS, 3. Other UEs | 3. GNSS #x, #y, 4. MEC #x | NEF APIs, MEC APIs, .. | Support for sidelink positioning |

Without this solution app server needs to consolidate all measurements and interact with different systems to get the required location. The profiling helps the optimization of the process based on the environment, UE context, etc., per profile checking/monitoring. Also, the configuration of combined positioning methods to meet the LCS requirements, is done with minimum exposure to the 3rd party/customer.

#### 7.5.1.2	Procedure

The procedure includes the translation of the vertical request to a Location profile and the procedures with the involved entities to derive the requested location report. The fused location function/client may also fetch location reports in an iterative manner based on method priorities, to ensure that the vertical requirement is met, with the minimum signalling/complexity.

Figure 7.5.1.2-1 illustrates a solution for the location profiling for supporting fused location exposure.

Pre-conditions:

1.	The VAL server has registered to receive fused location function services.

Figure 7.5.1.2-1: Location profiling for fused location derivation and exposure

1.	The fused location function configures a set of location service profiles, where each location service profile includes metrics like the positioning method, QoS parameters, location service producers involved, environment/ area type, etc.

NOTE1:	How the profiling attributes are selected/configured is up to implementation.

2.	The fused location function receives a location request from a fused location service consumer (VAL server), where this request may include a VAL server ID, location QoS requirements (accuracy, response time,..), location granularity (coordinates, cell-level, civic addresses, topological location), vertical specific support information (planned route, road maps,...), time validity for the requirement, area of validity, event triggering criteria (under which criteria the location report needs to be sent), etc

3.	The fused location enabler server determines a mapping of the fused location service consumer (VAL server) to a location service profile based on the location request and the information provided within the request.

4.	The fused location function sends to the fused location client and optionally the involved 3gpp functions the configuration of the mapping of the application to a Location profile. This may include the report configuration per Location profile (thresholds for event triggering, periodicity of reporting, format of reporting, minimum time between consecutive reports) as well as priority of positioning methods and location report granularity (coordinates, cell-level, civic addresses, topological location).

5a. The fused location function requests from the fused location client a local fused location estimate of the target VAL UE or for the UEs within the application in close vicinity.

5b. The fused location client sends to the fused location function a local fused location estimate based on the request.

6.	The fused location function performs a location request to one or more of the following (based on the Location profile):

-	to SEAL LMS, acting as VAL server, to request the location information for one or more UEs, as described in 9.3.5 of TS 23.434 (VAL triggered location reporting trigger). Such reporting trigger may also be enhanced to indicate the Location profile ID, as well as to indicate the locations of all the UEs with this Location profile in this area.

-	to GMLC directly or via NEF (see TS 23.273), acting as AF. The LCS service request is sent to GMLC or AMF, via NEF using the service-based interface or CAPIF API; or directly to GMLC if allowed to (e.g. fused location function is within MNO trust domain).

-	to 3rd party location servers as described in Solution #1.

NOTE2:	If SEAL LMS is used for the location retrieval, then the location retrieval from VAL UE and 5GC may happen via SEAL LMS.

7.	The fused location function calculates the fused location estimated based on combining location reports from previous steps, and may also perform additional processing e.g. for location augmentation, verification based on the Location profile. Then, it checks whether the estimate fulfils the Location profile requirement (based on QoS parameters such as accuracy, response time).

8-9.	If the requirement is not fulfilled, the fused location function iteratively requests further location information and re-checks whether the requirement is met.

10.	The fused location function sends the fused location report to the VAL server.

Editor's Note: The steps in this procedure may be impacted by the 5GFLS architecture considerations, and it is FFS to capture different variants based on different architectures.

### 7.5.2	Solution evaluation

Editor's Note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

## 7.6	Solution #6: Location service configuration

### 7.6.1	Solution description

This solution addresses key issue #5: Initialization and configuration for fused location service.

#### 7.6.1.1	Service flow for fused location service configuration

The high-level service flow for fused location service configuration is illustrated in figure below. This service flow is based on fused location architecture of solution#1.

Figure 7.6.1.1-1: Service flow for fused location service configuration

1.	The location service is initiated at the Fused location server. This can be triggered by an application through e.g. a service request or triggered by an event, by which the location service requirements for a target UE are identified.

2.	The fused location server may query the location capability of the target UE e.g. the location system or location service or location methods supported by the UE. The fused location server may need to decide the location source from which to receive location information based on the location service requirements as well as the target UE's location capability.

3a.	The fused location server may invoke the LCS service e.g. 5GC-MT-LR Procedure (as defined in 3GPP TS 23.273 [4]) by acting as AF or LCS client.

3b.	The fused location server may invoke the SUPL service e.g. network initiated flows (as defined in OMA AD SUPL [10]) by acting as the SUPL agent.

3c/d.	If the fused location server decides to receive location from the 3rd party location system, the fused location  server configures the location service with 3rd party location server to establish the secured data connection and session for location reports, and to support the location system configuration tailored for the location service requirements. The fused location server also interacts with fused location client of the target UE to inform the location service configurations.

### 7.6.2	Solution evaluation

Editor's note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

## 7.7	Solution #7: Location QoS based location sources and positioning methods selection

### 7.7.1	Solution description

This solution addresses key issue #2: Support of LCS QoS.

The Fused Location Fuction firstly needs to produce the fused location data from multiple sources based on the the rquested location QoS (e.g. the requirements of the positionting accuracy, reliability and latency). Based on the requested location QoS, the FLF needs to select one or more access types, one or more location methods (as described in TS 29.572 [14] ) and related CP/UP(SUPL) methods based on the requested location QoS (not exhausted):

-	2G/3G/4G/5G/NR satellite access

-	Non-3GPP access connected to 5GC

-	GNSS (e.g. GPS, Galilieo, BeiDou etc.)

-	Barometric Pressure

-	WLAN

-	Bluetooth

-	Terrestrial Beacon System (TBS) positioning based on MBS signals

-	Motion Sensor

-	RFID

-	Radio finger-print

-	Celll ID

-	ECID

-	OTDOA

-	DL_TDOA

-	DL_AOD

-	Multi-RTT

-	NR_ECID

-	UL_TDOA

-	UL_AOA

-	Ultra Wide Band (UWB)

-	Fingerprint

#### 7.7.1.1	Procedure of location QoS based location sources and positioning methods selection

Figure 7.7.1.1-1: location sources and positioning methods selection

1.	The application specific server sends a location request to the Fused Location Server to request the location information of the target Fused Location Client (UE Identity, location QoS). The location QoS can includes the location accuracy, reliability and latency etc. as described in clause 4.1b of TS23.273[4].

2.	The Fused Location Server queries the UE location context (e.g. in the internal database) with the location QoS to retrieve the available the access type, positioning methods as described in TS 29.572 [14] for the target Fused Location Client.

3.	The Fused Location Server selects the available access type (i.e. different location sources) and position methods to gets the UE location informations from these available access types.

4.	The Fused Location Server fuses the UE location information from different sources to get a fused UE location that meets the location QoS requirements.

5.	The FLS provides the fused UE location that meets the location QoS requirements to the application specific server.

Editor's note: Updates to the solution and solution evaluation based on Solution #1 is FFS.

### 7.7.2	Solution evaluation

This solution is based on the architecture proposed in Solution #1. Comparing to existing On-demand usage of location information in SEAL LM (TS 23.434[13], clause 9.3.9), the FLS can aggregate/fuse location data from more data sources so that more accurate location can be reported. Different location source can provide location information with difference location QoS, the FLS takes the different advantages of these difference location sources and decides to select which location sources based on the required location QoS and fuses the location information from the selected location sources to produce the final location information to meet the location QoS.

# 8	Overall evaluation

Editor's Note:	This clause will provide evaluations of the solutions.

# 9	Conclusions

## 9.1	General conclusions

Editor's note:	This clause will provide general conclusions for the study.

## 9.2	Conclusions of key issue #x

Editor's Note:	This clause will provide conclusions for the specific key issue.

Annex A: Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-07 | SA6#44-e | S6-211767 | - | - | - | Skeleton document proposal | 0.0.0 |
| 2021-07 | SA6#44-e |  |  |  |  | Implementation of the following p-CRs approved by SA6: S6-211768, S6-211769, S6-211771, S6-211772, S6-211792 | 0.1.0 |
| 2021-07 | SA6#44-e |  |  |  |  | Corrections of reference numbers in clause 4.1 and the bullet list in clause 5.1 | 0.1.1 |
| 2021-09 | SA6#45-e |  |  |  |  | Implementation of the following p-CRs approved by SA6: S6-212066, S6-212067, S6-212086, S6-212176 | 0.2.0 |
| 2021-10 | SA6#45-bis-e |  |  |  |  | Implementation of the following p-CRs approved by SA6: S6-212296, S6-212298, S6-212395, S6-212477 | 0.3.0 |
| 2021-11 | SA6#46-e |  |  |  |  | Implementation of the following p-CRs approved by SA6: S6-212704, S6-212706, S6-212759, S6-212828, S6-212836, S6-212837 | 0.4.0 |
| 2022-04 | SA6#48-e |  |  |  |  | Implementation of the following p-CRs approved by SA6: S6-220949, S6-220950 | 0.5.0 |
| 2022-05 | SA6#49-e |  |  |  |  | Implementation of the following p-CRs approved by SA6: S6-221459, S6-221460, S6-221461, S6-221487 | 0.6.0 |