| 3GPP TR 23.700-98 V1.1.1 (2022-07) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Enhanced architecture for enabling Edge Applications; (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	10

Introduction	11

1	Scope	12

2	References	12

3	Definitions of terms, symbols and abbreviations	13

3.1	Terms	13

3.2	Symbols	13

3.3	Abbreviations	13

4	Key issues	13

4.1	Key issue #1: Enhanced notification service to the EEC	13

4.2	Key issue #2: Enablement of Service APIs exposed by EAS	14

4.3	Key issue #3: Enhancements to service continuity planning	14

4.4	Key issue #4: EDGE-5	15

4.5	Key issue #5: Alignment of EDGEAPP and ETSI MEC	16

4.6	Key issue #6: Edge services support across ECSPs	16

4.7	Key issue #7: Application traffic filter exposure	17

4.8	Key issue #8: EAS selection synchronization	18

4.9	Key issue #9: Enhancement of dynamic EAS instantiation triggering	19

4.10	Key issue #10: Support for roaming UEs	19

4.11	Key issue #11: ACR between EAS and Cloud Application Server	20

4.12	Key issue #12: EEL service differentiation	20

4.13	Key issue #13: Edge enabler layer support for EAS synchronization	20

4.14	Key issue #14: Application traffic influence for initially selected EAS	21

4.15	Key issue #15: Support of constrained devices for Edge	21

4.16	Key issue #16: support of NAT deployed within the edge data network	22

4.17	Key issue #17: Discovery of a common EAS	22

4.18	Key issue #18: Linkage between EASs	23

4.19	Key issue #19: ACR scenario combination	24

4.20	Key issue #20: Method of supporting federated EAS service	24

4.21	Key issue #21: Simultaneously EAS connectivity in ACR	24

4.22	Key issue #22: EAS discovery in Edge Node sharing scenario	26

4.23	Key issue #23: Reliable Edge service	26

4.24	Key issue #24: SEAL capability access for EEL support	27

5	Architectural requirements	27

5.1	General requirements	27

5.1.1	General	27

5.1.2	Requirements	27

5.2	Enablement of Service APIs exposed by EAS	27

5.2.1	General	27

5.2.2	Requirements	27

5.3	ECS discovery	28

5.3.1	General	28

5.3.2	Requirements	28

5.4	Alignment of EDGEAPP and ETSI MEC	28

5.4.1	General	28

5.4.2	Requirements	28

6	Enhanced Application Architecture	29

6.1	Option #1: Roaming architecture	29

6.1.1	Architecture enhancements	29

6.1.1.1	Local breakout roaming architecture: Local breakout to access H-ECS	29

6.1.1.2	Home-routed EDGE-4 access to H-ECS	30

6.1.2	Identities	31

6.1.3	Cardinality rules	31

6.2	Option #2: Non-roaming architecture	31

6.2.1	Architecture enhancements	31

6.2.2	Identities	31

6.2.3	Cardinality rules	31

6.3	Option #3: Edge Notification Server architecture	31

6.3.1	Architecture enhancements	32

6.3.1.1	Edge Notification Server (ENS)	32

6.3.1.2	ENS Discovery	33

6.4	Option #4: Constrained devices with limited capabilities	33

6.4.1	Architecture enhancements	33

6.4.1.1	General	33

6.4.1.2	Architecture	33

6.4.2	Identities	34

6.4.3	Cardinality rules	34

6.5	Option #5: Architecture for ACR between EAS and CAS without CES	34

6.5.1	Architecture enhancements	34

6.5.2	Identities	35

6.5.3	Cardinality rules	35

6.6	Option #6: Architecture for ACR between EAS and CAS with CES	35

6.6.1	Architecture enhancements	35

6.6.2	Identities	36

6.6.3	Cardinality rules	36

6.7	Option #7: Architecture for Common EAS selection with central binding server	36

6.7.1	Architecture enhancements	36

6.7.2	Identities	36

6.7.3	Cardinality rules	36

6.8	Option #8: Architecture for ACR update in service continuity planning	37

6.8.1	Architecture enhancements	37

6.8.2	Identities	37

6.8.3	Cardinality rules	37

7	Solutions	37

7.0	Mapping of solutions to key issues	37

7.1	Solution #1: Service provisioning via push notification	39

7.1.1	Architecture enhancements	39

7.1.2	Solution description	39

7.1.2.1	General	39

7.1.2.2	Procedure	39

7.1.3	Solution evaluation	40

7.2	Solution #2: Traffic filter support for EDGE-3 API addressing application traffic detection	41

7.2.1	Architecture enhancements	41

7.2.2	Solution description	41

7.2.3	Solution evaluation	42

7.3	Solution #3: Service provisioning triggering via SMS over NAS	42

7.3.1	Architecture enhancements	42

7.3.2	Solution description	42

7.3.2.1	General	42

7.3.2.2	Procedure	42

7.3.3	Solution evaluation	44

7.4	Solution #4: ECS discovery through serving ECS to support edge services across ECSPs	44

7.4.1	Architecture enhancements	44

7.4.2.1	General	44

7.4.2.2	Procedure	45

7.4.3	Solution evaluation	46

7.5	Solution #5: ECS enhancement to discover EESs via other ECSs to support edge services across ECSPs	46

7.5.1	Architecture enhancements	46

7.5.2	Solution description	47

7.5.2.1	General	47

7.5.2.2	Procedure	47

7.5.3	Solution evaluation	48

7.6	Solution #6: ACR update in service continuity planning	48

7.6.1	Architecture enhancements	48

7.6.2	Solution description	48

7.6.2.1	ACR modification solution	49

7.6.2.1.1	EEC-based ACR modification procedure	49

7.6.2.1.2	EES-based modification procedure	50

7.6.2.1.3	ACR modification execution procedure	50

7.6.3	Solution evaluation	51

7.7	Solution #7: EES monitors UE mobility for service continuity planning	51

7.7.1	Architecture enhancements	51

7.7.2	Solution description	52

7.7.2.1	General	52

7.7.2.2	Procedure	52

7.7.3	Solution evaluation	54

7.8	Solution #8: EAS Service API enablement using CAPIF	54

7.8.1	Architecture enhancements	54

7.8.2	Solution description	55

7.8.2.1	General	55

7.8.2.2	CAPIF operations in Edge Enabler Layer	55

7.8.2.3	Service KPIs in CAPIF for EAS Service APIs	57

7.8.3	Solution evaluation	61

7.9	Solution #9: Application traffic influence trigger from EAS	61

7.9.1	Architecture enhancements	61

7.9.2	Solution description	61

7.9.2.1	General	61

7.9.2.2	Procedure	61

7.10	Solution #10: low power mode support	62

7.10.1	Architecture enhancements	62

7.10.2	Solution description	62

7.10.2.1	General	62

7.10.2.2	Procedure	62

7.11	Solution #11: A deployment option for alignment with ETSI MEC using CAPIF	63

7.11.1	Architecture enhancements	63

7.11.2	Solution description	63

7.11.2.1	General	63

7.11.2.2	Procedure	65

7.11.3	Solution evaluation	65

7.12	Solution #12: Service continuity planning permission	65

7.12.1	Architecture enhancements	65

7.12.2	Solution description	65

7.12.2.1	General	65

7.12.2.2	Procedure	65

7.12.3	Solution evaluation	66

7.13	Solution #13: Update ECS configuration information	67

7.13.1	Architecture enhancements	67

7.13.2	Solution description	67

7.13.3	Solution evaluation	67

7.14	Solution #14: V-ECS Discovery via the H-ECS	67

7.14.1	Architecture enhancements	67

7.14.2	Solution description	68

7.14.2.1	General	68

7.14.2.2	Procedure	68

7.14.3	Solution evaluation	69

7.15	Solution #15: Initial EAS selection declaration	70

7.15.1	Architecture enhancements	70

7.15.2	Solution description	70

7.15.3	Solution evaluation	71

7.16	Solution #16: EAS discovery for different users	71

7.16.1	Architecture enhancements	71

7.16.2	Solution description	71

7.16.2.1	General	71

7.16.2.2	Procedure	72

7.16.3	Solution evaluation	72

7.17	Solution #17: Traffic influence for initial EAS discovery	72

7.17.1	Architecture enhancements	72

7.17.2	Solution description	72

7.17.2.1	General	72

7.17.2.2	Procedure	72

7.17.3	Solution evaluation	73

7.18	Solution #18: Constraint device in EDGEAPP	73

7.18.1	Architecture enhancements	73

7.18.2	Solution description	73

7.18.3	Solution evaluation	75

7.19	Solution #19: EES determines the selected ACR scenario	75

7.19.1	Architecture enhancements	75

7.19.2	Solution description	75

7.19.2.1	General	75

7.19.2.2	Procedure	75

7.19.3	Solution evaluation	76

7.20	Solution #20: Propagation of EEL notifications to EEC using Edge Notification Server	76

7.20.1	Architecture enhancements	76

7.20.2	Solution description	76

7.20.2.1	General	76

7.20.2.2	Notification delivery over a direct Notification Channel Procedure	76

7.20.2.3	Notification delivery using a Push Server Procedure (indirect Notification Channel)	78

7.20.3	Solution evaluation	80

7.21	Solution #21: Prediction expiration time for service continuity planning enhancement	80

7.21.1	Architecture enhancements	80

7.21.2	Solution description	80

7.21.2.1	General	80

7.21.2.2	Procedure	81

7.21.3	Solution evaluation	82

7.22	Solution #22: Support simultaneous EAS connectivity in ACR	82

7.22.1	Architecture enhancements	82

7.22.2	Solution description	82

7.22.2.1	Solution for traffic influence	82

7.22.3	Solution evaluation	85

7.23	Solution #23: UE identification with NAT	85

7.23.1	Architecture enhancements	85

7.23.2	Solution description	85

7.23.2.1	General	85

7.23.2.2	Procedure	86

7.23.3	Solution evaluation	87

7.24	Solution #24: ACR between EAS and CAS with CES	87

7.24.1	Architecture enhancements	87

7.24.2	Solution description	88

7.24.2.0	General	88

7.24.2.1	ACR Scenarios	88

7.24.2.1.1	CAS decided ACR scenario	88

7.25.2.2.2	 "Discover T-EAS" for CAS	88

7.24.3	Solution evaluation	89

7.25	Solution #25: ACR between EAS and CAS without CES	89

7.25.1	Architecture enhancements	89

7.25.2	Solution description	89

7.25.2.1	General	89

7.25.2.2	Procedure	89

7.25.2.2.1	Updated 3GPP TS 23.558 clause 8.8.2.2 Initiation by EEC using regular EAS Discovery	89

7.25.2.2.2	Updated 3GPP TS 23.558 clause 8.8.2.3 EEC executed ACR via S-EES	91

7.25.2.2.3	Updated 3GPP TS 23.558 clause 8.8.2.4 S-EAS decided ACR scenario	93

7.25.2.2.4	Updated 3GPP TS 23.558 clause 8.8.2.5 S-EES executed ACR	95

7.25.2.2.5	EEC initiated ACR	97

7.25.2.2.6	CAS initiated ACR	97

7.25.3	Solution evaluation	98

7.26	Solution #26: Bundled EASs	98

7.26.1	Architecture enhancements	98

7.26.2	Solution description	98

7.26.2.1	General	98

7.26.2.2	Handling of EAS bundle information by the EEC	100

7.26.2.3	Handling of EAS bundle information and EAS bundle requirements by the EES	100

7.26.3	Solution evaluation	101

7.27	Solution #27: Enabling AC Association Aware services by selecting common EASs	101

7.27.1	Architecture enhancements	101

7.27.2	Solution description	102

7.27.2.1	General	102

7.27.2.2	New Information Elements	102

7.27.2.3	Enhancements to existing Information Elements	103

7.27.2.4	Procedure for determining common EES	103

7.27.2.5	Discovery of common EAS Procedure	103

7.27.2.6	ACR with common EAS Procedure	104

7.27.3	Solution evaluation	104

7.28	Solution #28: Common EAS discovery using EAS selection information	104

7.28.1	Architecture enhancements	104

7.28.2	Solution description	104

7.28.2.1	General	104

7.28.2.2	Procedure	104

7.28.3	Solution evaluation	106

7.29	Solution #29: Discovery of a common EAS	107

7.29.1	Architecture enhancements	107

7.29.2	Solution description	107

7.29.2.1	General	107

7.29.2.2	Procedure	108

7.29.3	Solution evaluation	110

7.30	Solution #30: Common EAS selection	110

7.30.1	Architecture enhancements	110

7.30.2	Solution description	110

7.30.3	Solution evaluation	112

7.31	Solution #31: Discover common EAS	112

7.31.1	Architecture enhancements	112

7.31.2	Solution description	112

7.31.2.1	General	112

7.31.2.2	Procedure	112

7.31.3	Solution evaluation	115

7.32	Solution #32: Dynamic EAS instantiation triggering and notification	115

7.32.1	Architecture enhancements	115

7.32.2	Solution description	116

7.32.2.1	General	116

7.32.2.2	Dynamic EAS instantiation triggering and notification procedures	117

7.32.3	Solution evaluation	118

7.33	Solution #33: Support for EEC Discovery of EAS(es) before instantiation	118

7.33.1	Architecture enhancements	118

7.33.2	Solution description	118

7.33.2.1	General	118

7.33.2.2	Procedure	119

7.33.3	Solution evaluation	120

7.34	Solution #34: EDGE-5 APIs	120

7.34.1	Architecture enhancements	120

7.34.2	Solution description	121

7.34.2.1	General	121

7.34.2.1	Procedure	121

7.34.2.2.1	General	121

7.34.2.2.2	AC registration request	121

7.34.2.2.3	EAS discovery request	122

7.34.2.2.4	ACR request	123

7.34.2.2.5	AC subscription request	124

7.34.2.2.6	AC notification	125

7.34.3	Solution evaluation	126

7.35.1	Architecture enhancements	126

7.35.2	Solution description	126

7.35.2.1	General	126

7.35.2.2	Procedure	126

7.35.3	Solution evaluation	127

7.36	Solution #36: Alignment of EDGEAPP and ETSI MEC	128

7.36.1	Architecture enhancements	128

7.36.2	Solution description	128

7.36.2.1	General	128

7.36.2.2	Alignment of EAS registration and MEC application registration	128

7.36.3	Solution evaluation	130

7.37	Solution #37: ACR request trigger timing	131

7.37.1	Architecture enhancements	131

7.37.2	Solution description	131

7.37.2.1	Procedure	131

7.37.2.2	Enhancements to procedures in TS 23.558	131

7.37.2.2.1	Enhancements to ‘Initiation by EEC using regular EAS Discovery’ in clause 8.8.2.2	132

7.37.2.2.2	Enhancements to ‘EEC executed ACR via S-EES’ in clause 8.8.2.3	132

7.37.2.2.3	Enhancements to ‘EEC executed ACR via T-EES’ in clause 8.8.2.6	133

7.37.3	Solution evaluation	133

7.38	Solution #38: ACR coordination	133

7.38.1	Architecture enhancements	133

7.38.2	Solution description	133

7.38.2.1	General	133

7.38.2.2	Procedure	134

7.38.3	Solution evaluation	135

8	Deployment scenarios	135

8.1	<Title>	135

9	Involved entities and relationships	135

9.1	Federation and Roaming	135

9.1.1	General	135

10	Overall evaluation	136

10.1	Architecture enhancements	137

10.1.1	Support for Terminal Equipment (TE)	137

10.2	Key issue evaluations	137

10.2.0	General	137

10.2.3	Key issue #3: Enhancements to service continuity planning	142

10.2.4	Key issue #4: EDGE-5	143

10.2.5	Key issue #6: Edge services support across ECSPs	143

10.2.7	Key issue #10: Support for Roaming UEs	144

10.2.8	Key issue #11: ACR between EAS and Cloud Application Server	147

10.2.9	Key issue #14: Application traffic influence for initially selected EAS	147

10.2.10	Key issue #18: Linkage between EASs	148

10.2.11	Key issue #21: Simultaneously EAS connectivity in ACR	148

10.2.x	Key issue #x	148

11	Conclusions	148

11.1	General	148

11.2	Conclusions for normative work	148

11.2.1	General conclusions	148

11.2.2	Architecture enhancement conclusions	149

11.2.3	Solution conclusions	149

Annex A (Informative): ETSI MEC and EDGEAPP system comparison	151

A.1	General	151

A.2	Service consumer and service provider	151

A.3	EAS/MEC application profile provisioning	151

A.4	EAS registration and EAS discovery	152

Annex B (Informative): Deployment and Evolution options of EDGEAPP and ETSI MEC platforms	153

B.1	General	153

B.2	Deployment options	154

B.2.1	Deployment Option-1: Collocated Platforms	154

B.2.2	Deployment Option-2: Converged architecture	154

B.2.2.1	General	154

B.2.2.2	Evolution Options	155

B.2.2.2.1	General	155

B.2.2.2.2	Evolution Option #1- Enhancement of a deployed MEP to support the functionality of EES	155

B.2.2.2.3	Evolution Option #2 Enhancement of a deployed EES to support the functionality of MEP	155

B.2.3	Deployment Option-3: non-Collocated Platforms	156

Annex C (informative): Change history	157

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

Rel-17 TS 23.558 [2] defines the application layer architecture to enable edge applications over 3GPP networks. It includes features such as ECS discovery, service provisioning, EAS discovery, EEC/EAS/EES registration, network and Edge Enabler Layer capability exposure, service continuity support with seamless service continuity and EEC context continuity etc., along with cardinality rules, deployment options, involved relationships and mapping with ETSI MEC [3] and GSMA OP [4] architectures.

This TR documents a study on architecture and procedure enhancements to improve the Rel-17 architecture for enabling edge applications, and to support emerging industry requirements.

# 1	Scope

The present document is a technical report capturing the study on enhanced architecture for enabling edge applications over 3GPP networks. The study bases the enhancements on the work done in 3GPP TS 23.558 [2] and takes into consideration other related work done within and outside 3GPP i.e. ETSI MEC [3] and GSMA OP [4].

The aspects of the study include identifying new key issues, architecture requirements, related architecture enhancements and solutions, cardinality rules, deployment options, and involved entities and relationships to enhance the Rel-17 architecture for enabling edge applications.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 23.558: "Architecture for enabling Edge Applications".

[3]	ETSI ISG MEC ETSI GS MEC 003, "Multi-access Edge Computing (MEC); Framework and Reference Architecture".

[4]	GSMA OPG.02: "Operator Platform Telco Edge Requirements", https://www.gsma.com/futurenetworks/wp-content/uploads/2021/07/GSMA-OPG-Telco-Edge-Requirements-2021.pdf

[5]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[6]	3GPP TR 23.758: "Study on application architecture for enabling Edge Applications".

[7]	3GPP TR 23.721: "Study on Sponsored Data Connectivity Improvements".

[8]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[9]	3GPP TS 23.203: "Policies and Charging control architecture; Stage 2".

[10]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[11]	3GPP TS 23.503: "Policies and Charging control architecture; Stage 2".

[12]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[13]	ETSI GS MEC 010-2: "Multi-access Edge Computing (MEC); MEC Management; Part 2: Application lifecycle, rules and requirements management".

[14]	ETSI GS MEC 011 v3.0.5: " Multi-access Edge Computing (MEC); Edge Platform Application Enablement".

[15]	ETSI GS MEC 001: " Multi-access Edge Computing (MEC); Terminology ".

[16]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs; Stage 2".

[17]	3GPP TS 29.522: "Network Exposure Function Northbound APIs; Stage 3".

[18]	3GPP TS 29.122: "T8 reference point for Northbound APIs".

[19]	3GPP TS 23.548: "5G System Enhancements for Edge Computing; Stage 2".

[20]	3GPP TS 28.538: "Management and orchestration; Edge Computing Management".

[21]	3GPP TR 22.944: "Report on service requirements for UE functionality split".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1], 3GPP TS 23.558 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1] and 3GPP TS 23.558 [2].

Cloud data network:  A data network having a set of servers deployed in a central place and capable of serving UEs from wider coverage area, as opposed to the edge data network.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1], 3GPP TS 23.558 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1] and 3GPP TS 23.558 [2].

Abbreviation format (EW)

<ABBREVIATION>	<Expansion>

# 4	Key issues

## 4.1	Key issue #1: Enhanced notification service to the EEC

The EEC can be notified with the updated information for edge computing service by EES and ECS over EDGE-1 and EDGE-4, respectively. The notification services available to the EEC (i.e., service provisioning notification, EAS discovery notification, ACR information notification) are specified in release 17, but the details on the notification mechanism is not addressed. In this regard, additional ways (e.g. push notification mechanism and application triggering specified in 3GPP TS 23.501 [5], etc.) of providing the updates to the EEC in real time need to be studied.

Open issues:

1.	Whether and how the EEC acquires the notification target address or a notification channel URI to receive the notifications?

2.	Whether and how EEC, ECS and EES support push notification mechanism. Whether and what additional functional entity is necessary for this?

3.	How are the EEC subscriptions and/or notification targets treated in mobility scenario (e.g. during ACR scenario)?

4.	Whether and how to utilize application triggering method specified in 3GPP TS 23.501 [5] to provide notifications to the EEC?

## 4.2	Key issue #2: Enablement of Service APIs exposed by EAS

As specified in 3GPP TS 23.558 [2] (Rel-17), the Edge Enabler Layer exposes Service APIs towards the EASs. The exposed Service APIs include the capabilities provided by EES as specified in the clause 8.6 of 3GPP TS 23.558 [2] (Rel-17) and the capabilities provided by the 3GPP core network as specified in the clause 8.7 of 3GPP TS 23.558 [2] (Rel-17).

However, there are several use cases and requirements to exploit Service APIs exposed by EASs (provided by Application service providers) which can be invoked by the other EASs such as Cloud/Edge/Split Rendering in AR/VR use cases; and video content delivery exploiting separate services such as content caching, video encoding/decoding, and video analytics.

With the enablement of Service APIs exposed by EASs in the Edge Enabler Layer, there may expect several benefits to the involved business roles in the edge computing services. For example, Application service providers may simplify the Edge Application logic by composing the service components provided by the other EASs at the network edge; and open a new business to provide one or more service components to be executed and invoked by the other EASs at the network edge.

In order to enable the use of Service APIs exposed by EASs in the Edge Enabler Layer, there have been relevant studies in 3GPP TR 23.758 [6] (Rel-17), which result in Sol#15 as follows:

-	Solution #15: Edge Application Server's service APIs publish and discovery using CAPIF:

-	The Edge Enabler Server can support edge application (owned by 3rd party or by PLMN operator) access to the service APIs offered by other Edge Application Servers within and across the Edge Data Network by providing CAPIF functions in a distributed or centralized manner.

Based on the Rel-17 study result, this key issue focuses on addressing the following open issues.

Open issues:

1.	Identify any gaps in CAPIF to enable EAS Service APIs in the EDGEAPP architecture based on the Sol#15 of 3GPP TR 23.758 [6] (Rel-17).

-	In terms of e.g., service-specific attributes for API publish/discovery, API availability subscription/notification across EES/CCF

2.	If any, whether and how to enhance CAPIF capabilities to address the gap identified as above?

## 4.3	Key issue #3: Enhancements to service continuity planning

In 3GPP TS 23.558 [2], the service continuity planning was specified as part of the Edge Enabler Layer value-add features for supporting seamless service continuity, when information about planned, projected, or anticipated behaviour is available at EESs or provided by EECs.

This key issue studies potential enhancements to the service continuity planning feature, based on additional criteria for detecting a planned ACR, e.g. the network conditions / analytics monitoring, the DN performance monitoring, the expected/predicted UE route etc. This key issue also studies potential enhancements to the service continuity planning feature to allow the EEC to send a timely ACR request. Finally, the key issue studies scenarios when the planned UE behavior changes after the launch of the service and the ACR needs to be modified due to these changes, e.g. due to UE mobility change.

Open Issues:

1.	How to rely on the capability of EES/EEC to detect whether the UE moves to the predicted location or not for service continuity planning?

2.	Whether and how the EEL can support the determination of the ACR request trigger timing in case of service continuity planning?

3.	How to deal with scenarios when the ACR needs to be modified, e.g. due to UE mobility?

4.	Whether and what additional capability exposure is required from the 5GS (e.g. NWDAF, OAM) to enhance the service continuity planning?

5.	Potential impact on information exchanged between EAS and EEL.

6.	Potential impact on information to communicate within the EEL.

## 4.4	Key issue #4: EDGE-5

According to the latest version of the EDGEAPP specification in 3GPP TS 23.558 [2]:

"EDGE-5 reference point enables interactions between AC(s) and the EEC."

That specification also notes:

"NOTE:	Detailed specification of this reference point is out of scope of this release of this specification."

According to the latest version of the FS_EDGEAPP specification in 3GPP TR 23.758 [6]:

"The interactions between Application Client(s) and the Edge Enabler Client in the UE are supported by the EDGE-5 reference point. This reference point supports:

-	Obtaining information about Edge Application Servers that Application Client require to connect;

-	Notifications about events related to the connection between Application Clients and their corresponding Edge Application Servers, such as: when an Application Client needs to reconnect to a different Edge Application Server;

-	Providing Application Client information (such as its profile) to be used for various tasks such as, identifying the appropriate Edge Application Server instance to connect to; and

-	Provide the identity of the desired Edge Application Server to the Edge Enabler Client to enable it to use that identity as a filter when requesting information about Edge Application Servers."

The necessary functionality of the EDGE-5 interface needs to be studied for Release 18:

-	What functionality should an EEC provide to an AC (see the interactions copied from 3GPP TR 23.758 [6] and the procedures and information flows specified in 3GPP TS 23.558 [2])?

The following aspects may also need to be studied for Release 18 based upon the provided functionality of the EDGE-5 interface:

1.	Whether the cardinality as currently captured in 3GPP TS 23.558 [2] is to be modified:

"The following cardinality rules apply for EDGE-5:

a)	One AC may communicate with only one EEC; and

b)	One EEC may communicate with one or more AC(s) concurrently."

2.	Whether and how an AC can discover available EEC(s)?

3.	Whether mutual authentication and authorization between an AC and an EEC is necessary, and if so, how is that accomplished?

4.	What APIs should be exposed from an EEC to an AC to support that functionality?

5.	Whether a notification mechanism is necessary from an EEC to an AC?

6.	Whether and how an AC registers to an EEC?

7.	Whether and how an AC de-registers from an EEC?

8.	Whether and how an AC detects an abnormal termination of an EEC?

9.	Whether and how an EEC detects an abnormal termination of an AC?

10.	Whether user's consent is necessary to either AC or EEC operation, and if so, how is it provided?

11.	Whether and how can EDGE-5 support constrained devices with limited capabilities (such as a terminal equipment as defined in TR 21.905) over EDGE-5?

NOTE 1:	The aspects of defining end-user consent/authorization over APIs and aspects of mutual authentication and authorization between an AC and an EEC are in the scope of SA3.

NOTE 2:	The aspects of the usage of end-user consent/authorization over APIs is in the scope of SA6.

## 4.5	Key issue #5: Alignment of EDGEAPP and ETSI MEC

As described in Annex C of 3GPP TS 23.558 [2] (Rel-17), both EDGEAPP and ETSI MEC can provide support for hosting different edge applications. According to Annex C: "Both EAS and MEC application are application servers and can provide similar application specific functionalities. EAS utilizes the services of EES as specified in this document whereas MEC application utilizes the services provided by MEC platform as specified in ETSI GS MEC 003." As discussed in Annex B.2 of draft GS MEC 003 [3] (v3.0.4), the EES and MEC platform can also be collocated in an implementation.

While the 3GPP TS 23.558 [2] and ETSI GS MEC 003 [3] provide an initial view about the alignment of the two platforms, this KI intends to address the following:

1.	Study and analyse different deployment options of EDGEAPP and ETSI MEC platforms.

2.	Functional architecture and gap analysis between EDGEAPP and ETSI MEC to determine complementary and possibly overlapping APIs and other related functionalities. Annex A captures a comparison of the architectures to facilitate the gap analysis.

3.	Recommendation and enhancements based upon the outcome of (1) and (2).

NOTE 1:	Backward compatibility is an important aspect of any recommendations & enhancements and will be considered during the study of this KI.

NOTE 2: This KI is limited to alignment aspects within 3GPP capabilities.

## 4.6	Key issue #6: Edge services support across ECSPs

An edge service or an EAS (e.g. V2X server) can be provided via different EDNs deployed by different ECSPs. Each ECSP may not have the required infrastructure to install the EAS in every EDN due to financial, regulatory and operation constraints. It is assumed that a UE can access the same edge service served by different EASs which are registered to different EESs and deployed by different ECSPs, which have a service level agreement to share edge services. These ECSPs can deploy EESs to serve different PLMNs or different coverages of the same PLMN. A typical scenario is depicted in Figure 4.6-1.

Figure 4.6-1 EAS deployed by different ECSPs

In Figure 4.6-1, the EAS2 resident in EDN1 and EDN2 provides the same service. The UE may be configured with the ECS1 configuration information (e.g., if the UE is a subscriber of ECSP_1). It is not clear how to provision the ECS2 configuration information, deployed by ECSP_2 (a partner of ECSP_1), to the UE when the UE is out of the service area of EAS2 in ECSP1 and cannot find a suitable EES within ECSP1 to discover and connect to EAS2.  The same issue exists when EAS2 becomes unavailable due to other reasons, e.g. overload, or in cases where ECSP_1 does not deploy EAS2 at all and relies on partner ECSP_2 to provide the edge service.

Besides, the UE may have already accessed the EAS2 in the EDN1 and is getting service from that EAS. In that case, it is not clear how to support service continuity due to UE mobility when the UE moves out of the service area of the EAS2 in EDN1 and goes to the service area of the EAS2 in EDN2.

Furthermore, the target EDN and source EDN are operated by different ECSP which may not have SLA with each other, then the S-EES may not be able to communicate with a T-EES (discovered from ECS) due to lack of SLA. Unfortunately, in Rel17 this failure may only be detect upon EDGE-9 interaction.

The following study is needed:

1.	Identify potential enhancements to the existing architecture defined in Rel-17 to enable inter-ECSP interactions.

2.	Study potential impact to support ECS discovery and service provisioning based on UE location.

3.	Whether and how EEC registers with an EES deployed by a partner ECSP?

4.	Study potential impact to support service continuity.

5.	How is EEC context continuity maintained across ECSPs with or without ACR?

6.	How the ECS can discover a T-EES having SLA with S-EES based on the federation agreements between ECSPs before EDGE-9 interaction?

## 4.7	Key issue #7: Application traffic filter exposure

3GPP TS 23.558 [2] has specified EDGE-3 exposure with different APIs. The session with QoS API provides the capability for the EAS to influence the QoS for the application traffic via EES. The ACR management event API supports "User plane path change", "ACR monitoring" and "ACR facilitation" and all events support to detect user plane path change for the application traffic.

The current definition of session with QoS API and ACR management event API only supports simple IP flow description. E.g. in 3GPP TS 23.558 [2], table 8.6.6.3.2-1 IP flow description identifies the application traffic by 3 or 5 tuples.

Table 8.6.6.3.2-1: Session with QoS create request

| Information element | Status | Description |
| --- | --- | --- |
| EASID | M | The identifier of the EAS |
| Security credentials | M | Security credentials of the EAS |
| UE IP address (NOTE 1) | O | The UE IP address. |
| UE ID (NOTE 1) | O | The identifier of the UE (i.e. GPSI) |
| UE Group ID (NOTE 1) | O | Identifies a group of UEs (i.e. internal group ID or external group ID) |
| IP flow description | M | The IP flow description for the application traffic. |
| Requested QoS reference (NOTE 2) | O | Refers to pre-defined QoS information for the data session between AC and EAS (NOTE 3). |
| … | … | … |

Only supporting IP flow description in EDGE-3 exposure APIs is not enough. EPS already supported the application traffic (e.g. encrypted application traffic) detection by more filters than IP flow description for the application (e.g. considering in Rel-14 study 3GPP TR 23.721 [7]) and more traffic filters are supported via PFD management procedure as described in 3GPP TS 23.682 [10] and 3GPP TS 23.203 [9], and such capability is also supported in 5GS in 3GPP TS 23.502 [8] and 3GPP TS 23.503 [11] correspondingly. The EEL should provide the same level of traffic filter for identifying the application traffic.

Open issues:

1.	How to support more application traffic filter for session with QoS API.

2.	How to support more application traffic filter for ACR management event API.

## 4.8	Key issue #8: EAS selection synchronization

Currently, EAS discovery may result in multiple EASs being discovered for a specific AC. The discovery request may trigger at the EES operations such as dynamic instantiation (3GPP TS 23.558 [2] clause 8.12). However, the EEC may select only one or some of the discovered EASs to enable AC communications or may not begin communications right after a discovery. For a discovered but unselected EAS, operations such as EAS instantiation or state change are unnecessary and inefficient,

From a different perspective, the registration procedure results in EEC context establishment at EES, with associated service session management. The EES process for determining EAS selection for service sessions is not specified, and there is no method to synchronize this information with the EEC. Moreover, following a registration the EEC does not have EAS endpoint information which could be leveraged for initiating service sessions, although the registration results in reservation of resources in the EDN.

In another case, many IoT devices are configured for bursty communications at large intervals of time, without service continuity requirements and with many semi-static (e.g. provisioned over-the-top) parameters. For example, a set of EASs may be maintained at the EEC with the expectation of providing signalling optimizations. However, this EEC information cannot be currently leveraged by the system as intended.

While basic edge functionality can be enabled assuming ideal implementations of an algorithm through which the EES determines that a registered EAS is providing services to an AC, inefficiencies in EAS capability use and enabling service session functionality immediately after registration remain. Hence, it is required to study:

1.	How to enable the EES to accurately determine the EAS(s) capabilities needed by EEC for service sessions in order to perform optimal EAS instantiation operations.

2.	Whether and how the service session communications between ACs and EASs can be enabled by the EEC as soon as the EDN capabilities are available after registration.

3.	Whether and how to enable the EES to leverage pre-existent EAS information at the EEC in order to enable service session communications efficiently for IoT devices.

## 4.9	Key issue #9: Enhancement of dynamic EAS instantiation triggering

In order to ensure efficient utilization of EDN resources for EAS deployment, it should be possible to have the proper number of EAS instances in the EDN to accommodate the load for applications. The dynamic EAS instantiation triggered by the EES is supported in release 17, but further details are not addressed. The EES may invoke EAS dynamic instantiation triggering to the EAS management system, e.g,, for considering the service load/capacity of EAS (e.g., number of service session); and for considering the EEC's requesting service characteristics (e.g., location, latency). In this regard, the followings need to be studied further.

Open issues:

1.	What kind of information can be acquired by edge enabling layer and utilized by an EES to decide to trigger dynamic EAS instantiation and which entities can provide such information to an EES

2.	Whether and how to support dynamic EAS termination triggering in order to enable dynamic scaling of EAS (i.e., scale in as needed).

NOTE:	The aspects of the interaction between EES and EAS management system is in the scope of SA5.

## 4.10	Key issue #10: Support for roaming UEs

When a UE is roaming in VPLMN, EDN configuration information for edge computing service in VPLMNs may not be available at all ECS deployed in HPLMN (termed H-ECS. The EEC in the UE thus needs to obtain information for V-ECSs (ECS available in VPLMN) to obtain service provisioning information in VPLMN based on the business relationship between HPLMN operator, VPLMN operator, and related ECSP(s).

In one scenario, the EEC in the roaming UE needs to discover the availability of edge computing services via ECS(s) available in VPLMN.

In Rel-17, ECS discovery based on VPLMN ID addressed in clause 8.3.2 of 3GPP TS 23.558 [2] do not cover some cases, e.g. when there are multiple available ECSs via the VPLMN for the roaming UE. Therefore, it is required to study ECS discovery in VPLMN and subsequent service provisioning for all relevant deployment models.

Additionally, it is required to clarify how an EEC hosted in the roaming UE can be authenticated and authorized to access the edge computing services available in the VPLMN. The related requirement is described in GSMA OPG as follows: "Access of roaming subscribers to edge applications in the visited network shall be subject to authorisation by the subscriber's Home OP and the Visited OP". Note that EEC authentication/authorization in Rel-17 is not clarified in roaming situation. It is thus required to study the architectural support necessary for SA3-defined procedures for EEC authentication/ authorization in roaming scenarios.

The following aspects shall be studied to support roaming UEs:

1.	Roaming-related deployment scenarios (if any) to be supported, and which may be differentiated by: (i) the relationship between the ECS provider and the PLMN operators, (ii) whether connectivity to an ECS can be established in both involved PLMNs.

2.	How the EEC in the roaming UE knows the availability of ECS(s) and/or EES(s) and discovers them in the VPLMN?

3.	Whether and how edge computing service continuity is supported when transitioning between an HPLMN and VPLMN.

4.	How to support authentication and authorization for an EEC hosted in the roaming UE.

5.	Whether and how to support topology hiding on inter-PLMN/ECSP interfaces on the edge enabler layer.

## 4.11	Key issue #11: ACR between EAS and Cloud Application Server

When a UE moves to a new location, different EASs or Cloud Application Server (CAS) can be more suitable for serving the ACs in the UE. Such transitions can result from a non-mobility event also, requiring support to maintain the continuity of the service.

This key issue is to support service continuity for ACs in the UE to minimize service interruption while switching the application server between Edge and Cloud. To support service continuity, the application context is transferred between EAS and CAS.

Rel-17 Edge Computing work is limited to the service continuity between the EAS(s) and identified several scenarios for service continuity. Detailed study is required to enable service continuity between EAS and CAS, covering the following open issues:

1.	Whether and how to detect that ACR is required between EAS and CAS

2.	Whether and how to decide that ACR is required between EAS and CAS

3.	Whether and how to perform ACR between EAS and CAS

4.	Whether and how to perform post ACR actions

5.	Whether EEL is required on the cloud deployment and what are the potential impacts to the CAS architecture

6.	Whether and what are the impacts of CAS initiated ACR.

## 4.12	Key issue #12: EEL service differentiation

The service differentiation in Rel-17 is very general and not detailed (e.g. ECS use local policy to determine service provisioning).  Details to enable an ECSP to provide different service quality levels should be specified. Such as a principle based on the role of service consumer. For example, a premium user must get the nearest available edge, or there should be certain applications available only to the premium users and so on.

Open issues:

-	What service differentiation should be enabled by the EEL?

-	Which procedure should be enhanced to support service differentiation?

-	Which functional entity is responsible for defining or managing the service differentiation?

## 4.13	Key issue #13: Edge enabler layer support for EAS synchronization

It is possible for the ASP to provide EAS (with same service) in different EDNs. For example, a gaming service provider may have deployed game servers in different EDNs to serve its users. It is required for an EAS to synchronize the particular communication session among other EAS(s). In figure 4.13-1, consider:

1)	a multi-user gaming session-1 is active between 5 gaming users where 2 users are in EDN1 served by EAS-1X (ASP1), while other 3 users are in EDN2 served by EAS-2X (ASP1);

2)	at same time (when gaming session-1 is active), another gaming session-2 is also active between different set of users, where some users are in EDN2 served by EAS-2X (ASP1), while other users are in EDN3 served by EAS-3X (ASP1).

It is required for EAS(s) to synchronize among each other for specific communication session (e.g. multi-user game session) in order to serve the users properly. It is required to study how Edge enabler layer can support EAS from EDN1 to find other EAS(s) in other EDN(s) to synchronize with.

Figure 4.13-1 EAS (with same service) deployed by same ASP in different EDNs

Further, referring to 3GPP TS 23.558 Annex A.4, the application architecture supports SEAL application server functions and Application Enabler Server functions available at the edge. While SEAL application server functions can be made available as an EAS at the edge, it is also possible that certain SEAL application server functions are available either or both at the edge and at the cloud. When the server functions of an application are available both at the edge and at the cloud, there may be a need for interaction between the two corresponding application servers.

This key issue is to support interaction between the two application server functions deployed at the edge or both at the edge and at the cloud.

Open issues:

1)	Whether and how to enable EAS to find other EAS(s) with multi-user communication session to synchronize?

2)	Whether and how to enable EAS to find other interested EAS(s) with specific service to synchronize?

3)	Whether and how to enable EAS to discover and interact with another application server function deployed on the cloud for context synchronization?

4)	Whether and how edge enabler layer could provide support to EAS synchronization?

## 4.14	Key issue #14: Application traffic influence for initially selected EAS

The application traffic is between the AC and EAS and 3GPP CN provides the underlying connectity for the application traffic. The EEC starts with EAS discovery first in order to offer EAS(s) to AC to start communication with the initially selected EAS. Later on, if the EAS relocation criteria is met, the EAS relocation procedure happens, which consists of EAS discovery, application traffic influence, application context transfer and AC communication with the new EAS.

Currently, it is not clear in the specification how to influence the application traffic with best optimal user plane routing when initial EAS discovery is completed in the application layer.

Open issues:

-	Whether and how the EEL can be involved to influence the application traffic routing in AC communication with the initially selected EAS.

-	Whether and how EEL configuration information that may influence application traffic routing.

-	Which entities can configure or provide information for application traffic influence routing.

## 4.15	Key issue #15: Support of constrained devices for Edge

Energy efficiency requirements are relevant for battery driven low-power IoT devices. Those devices may require edge computing services in a local DN for low-latency and employ the SA6 specified edge enablers.

Clause 6.15 of 3GPP TS 22.261 [12] has provided requirements for energy efficiency for devices, for instance: the 5G system shall support UEs using small rechargeable and single coin cell batteries; and shall support mechanisms to improve battery life for a UE over what is possible in EPS.

In the existing EDGEAPP architecture as described in TS 23.558 [2], many procedures and services are defined for EEC to interaction with EES/ECS. It is needed to study what impact the support for constrained UE may have on EDGEAPP architecture and what procedure can be improved.

Open issues:

-	Whether there are any impacts on the EDGEAPP architecture for constrained UE.

-	Whether and how the existing EDGEAPP architecture and procedures, for constrained UE to network communication (i.e. EDGE-1 and EDGE-4), e.g. can be improved to reduce power consumption.

## 4.16	Key issue #16: support of NAT deployed within the edge data network

In operational deployments of cloud infrastructures including at the edge, Network Address Translation (NAT) are oftentimes deployed.

Such an operational constraint will prevent the EES to have an IP address of the UE that is known by the underlying 3GPP network.

Open issue:

-	How the EES can access 3GPP network services pertaining to a UE when the edge data network employs Network Address Translators (NATs).

-	How AF specific and temporary UE IDs can be managed at the Edge Enabler Layer?

Editor's Note: Whether EAS requires to be addressed in the key issue is FFS

## 4.17	Key issue #17: Discovery of a common EAS

An ASP can deploy several EASs providing the same service in different locations within the EDN.

For certain use cases involving real-time communication in a multi-user session, both between AC and EAS and between different ACs via the EAS, it may be necessary or beneficial to use services from a single common EAS to meet the strict latency requirements and to avoid the need for inter-EAS synchronization. The use cases may include, for example, a team of robots coordinating together on a manufacturing floor, a team of surgeons using VR headsets and robotic surgery equipment to operate together on a patient, or a group of trucks using V2X for platooning.

Dependent on the use case, the EEL may apply different additional criteria to determine this common EAS. E.g. it could be desirable to determine the EAS so that the latency for all the ACs in the session is approximately the same or that the latency for a specific AC is minimized.

Figure 4.17-1 Several EASs (with same service) deployed in different locations in the same EDN

Open issues:

1)	Whether and how the ACs/EECs of different users can select or be provisioned the same EAS within an EDN?

NOTE:	This open issue is dealing with the issue how different EECs can perform EAS discovery so that they select the same EAS within an EDN, whereas KI#13 is dealing with the issue how, after different EECs have selected different EASs located in different EDNs, these EASs can synchronize their contexts.

2)	Whether and how the ACs/EECs of different users can select or be provisioned a common EAS, even if initially the EECs are communicating with different EDNs?

3)	Whether and how the EEL can support service continuity to ensure that when ACs require the use of service from a common EAS and an ACR operation is needed, ACR operations can be coordinated so that upon completion of the ACR operations the ACs again have services provided by a common EAS.

## 4.18	Key issue #18: Linkage between EASs

Rel-17 EEL procedures are designed such that services like EAS discovery and service continuity support are performed per EAS, where individual EASs are uniquely identified using EAS endpoints (e.g., URI, FQDN, IP address).

However, to provide services to the end user a typical AC communicates with multiple endpoints i.e., multiple EASs. This creates a linkage between these EASs, which impacts the support provided by the Edge Enabler Layer. Taking an example of an online game where to support large number of users, different game functions are split across multiple servers; like, a game engine for game state and user input management, in-game chat server for communication between players and a capture server for capturing rendered images, encoding, and transporting them to the player's device. If each of these EASs are discovered, controlled, and relocated individually, it may impact the overall quality of service. For e.g. ACR failing for the game engine should cancel the ACR of the capture server to maintain their proximity.

This key issue is to study the impacts on the EEL's support functions created by such linkages or dependencies between the EASs.

NOTE:	This KI does not focus on enabling communication between the EASs, rather, it focuses on coordination at the EEL.

Open issue:

-	How can the EEL identify linkages between the EASs?

-	What are the impacts on EEL procedures due to linkage between the EASs e.g. when the linked EASs are served by the same EES and require ACR due to UE mobility?

## 4.19	Key issue #19: ACR scenario combination

Multiple ACR scenarios are specified in 3GPP TS 23.558[2] clause 8.8. Applications can utilize one or more ACR scenarios. Different combinations of utilizing ACR scenarios by Applications should be enabled by the Edge Enabler Layer (e.g. only one ACR scenario allowed or several ACR scenarios allowed).

Open issues:

-	Whether and how the EEL can support the determination of the ACR scenario for one AC?

## 4.20	Key issue #20: Method of supporting federated EAS service

In order for EAS to provide services (weather, transportation, maps, etc.) in partnership with other EASs, EAS context processing and federated EAS support may be required at edge-compatible layers. When ACR occurs due to UE mobility, a method of rearranging the federated EAS context may be required to provide continuous service of the federated EAS. In addition, there may be a need for a method for finding an EAS that provides a federated EAS service within the EDN in which the UE has moved.

Although EAS can discover and communicate other EAS APIs through CAPIF's functions, but for service continuity, it may be necessary to discover EASs providing federated EASs and relocation the context of EASs that provided federated capabilities.

NOTE:	This KI focuses on coordination at the EEL when the federated EAS provides services to the AC on a UE.

Open issues:

-	Whether and how the EEL can support to federated EAS context management.

-	Whether and how the EEL can support the relocation of the federated EAS context for service continuity.

-	Whether and how the EEL can discover EAS that provides the services of the federated EASs.

## 4.21	Key issue #21: Simultaneously EAS connectivity in ACR

In 3GPP TS 23.548 [19], there is a use case where application client needs to connect to both S-EAS and T-EAS during service continuity. Details are specified in clause 6.3.4 and Annex F of 3GPP TS 23.548 [19].

For AC triggered application context relocation, clause 8.8.2.2 of 3GPP TS 23.558 [19] describes the scenario for the service continuity initiated by EEC using regular EAS discovery. In step 5, the EEC and AC jointly decides the T-EAS to be used and in step 8 the AC is triggered by the EEC to start ACT.

Figure 4.21-1: ACR initiated by the EEC and ACs

Figure 4.21-1 illustrates one ACR scenario example in SA6 EDGEAPP. If the AC needs to connect to the T-EAS first to trigger ACT, the AC has two application sessions simultaneously during the service continuity. There could be more ACR scenarios applicable for this type of ACT. There are potential improvements in EEL to facilitate such simultaneous connectivity. For example, whether and how to influence the application traffic to maintain both S-PSA and T-PSA during the service continuity and the feature interaction between simultaneous EAS connectivity and service continuity. NOTE:	In Rel-17, EES can optimize the user plane routing before AC starts the application signalling with the T-EAS, but that does not consider the need for simultaneous EAS connectivity.

Open issues:

-	Whether and how EEL can influence the application traffic routing considering the need to maintain both S-PSA and T-PSA for simultaneous connectivity with both S-EAS and T-EAS, during the service continuity.

-	What the feature interaction is for simultaneous EAS connectivity and service continuity (including planning).

## 4.22	Key issue #22: EAS discovery in Edge Node sharing scenario

Based on OPG.02 CR1001 Operator Platform Telco Edge Requirements, Edge Node sharing scenario has been identified in GSMA OPG.02 CR1001 Operator Platform Telco Edge Requirements clause 3.3.5.

The deployment case is as follows:

1.	OP B deploys application in the OP A (partner OP). OP B wants to scale its services for the region covered by OP A by using OP A's edge infrastructure.

2.	User belongs to the OP B.

3.	If OP B finds that the most suitable application that can serve the user is available in OP A (partner OP), then OP B requests the edge computing service from OP A (partner OP).

NOTE 1:	The user is referred to the subscribers who have edge service authorizations.

Based on the deployment case, it is not clear how to discover and determine the EAS(s) deployed in OP A for OP B users.

Figure 4.22-1 Edge Node sharing scenario

The following study is needed:

1.	How can EES discover and determine a EAS which allows (subscribers of OP B) to avail its services?

NOTE 2:	The key issue assumes OP A and OP B has the same Edge Computing Architecture (i.e. EDGEAPP).

## 4.23	Key issue #23: Reliable Edge service

When the Edge services are deployed in a virtualized environments (i.e. built for cloud), it is expected that the overall reliability of the system shall be at least the same as the reliability of non-virtualized system. Therefore, the EDGEAPP service-based architecture as depicted in figure 6.2-1 of 3GPP TS 23.558 [2] should be designed in a way that seamless replacement, addition or removal of services is possible and does not require specific (re-)configuration of both the running and the new component(s).

The ECS/EES may experience unexpected events (e.g. hardware/link issue in nature disaster) and expected events (e.g. graceful shutdown for maintenance) in service.

The highly reliable edge computing aims to provide fault tolerance, high availability and service resilience for the application services as well as the application supporting layer.

This KI focuses on how edge computing (EDGEAPP in SA6) can provide high reliability in EES/ECS and to support highly reliable EAS in SA6 application layer.

NOTE:	SA5 is responsible for the management of 3GPP functions including edge entity LCM and the interactions with ETSI NFV MANO. This is, however, not in the scope of this KI.

Editor's note:	Whether the study result can be applied also to other application services (e.g. SEAL services) specified in SA6 is FFS.

Open issues:

-	Whether and what mechanisms the EES/ECS can use for high reliability in EES/ECS services during expected events and unexpected events in the service.

-	Whether and what mechanisms the EES/ECS can provide to support highly reliable EAS during expected events and unexpected events in the service.

NOTE: The reliability mechanism should make the network changes due to events transparent to the UE.

## 4.24	Key issue #24: SEAL capability access for EEL support

Annex A.4 of 3GPP TS 23.558 describes deployments of EES with SEAL services and Application Enabler Services as consumers. Based on this description, EES capabilities are available to SEAL servers over the EDGE-3 refernce point.

Annex A.4.2 of 3GPP TS 23.558 describes deployments of SEAL services at the edge, with EASs as consumers. For this case, the re-exposure of SEAL services is designed based on CAPIF implementation by EES.

EESs may also be a consumers of SEAL services such as location reporting, group managemnt, event monitoring, etc. which are currently exposed via SEAL-S and SEAL-E refernce points. New services under development (e.g. ADAES) may also be adopted as SEAL services and may be both consumers and producers of services at the EES.

Open issues:

-	How EEL accesses and utilizes SEAL capabilities deployed within the EDN.

# 5	Architectural requirements

## 5.1	General requirements

### 5.1.1	General

This clause provides a basic description of the general architectural requirements.

### 5.1.2	Requirements

This clause lists the general requirements.

## 5.2	Enablement of Service APIs exposed by EAS

### 5.2.1	General

This clause specifies the requirements for EAS Service API enablement in the EDGEAPP architecture to address the KI#2 in the clause 4.2.

### 5.2.2	Requirements

The following are the architectural requirements to support EAS Service APIs in the EDGEAPP Rel-18 architecture.

1)	EAS capability exposure

-	The application layer architecture shall support exposure of EAS's capabilities to the other EASs.

2)	EAS Service API publication

-	The application layer architecture shall support EAS to publish its exposing Service API information to EES

-	The application layer architecture shall support EAS to update the published EAS Service API information on the EES.

-	The application layer architecture shall provide mechanisms for the EAS to publish and update KPIs of its Service APIs when available.

3)	EAS Service API discovery

-	The application layer architecture shall provide mechanisms for an EAS to discover available EAS Service APIs.

4)	Subscription service

-	The application layer architecture shall provide subscription and notification mechanisms enabling an EAS to receive changes in dynamic information of EAS Service APIs from an EES.

-	The application layer architecture shall provide subscription and notification mechanisms enabling an EAS to receive changes in availability of EAS Service APIs from an EES.

## 5.3	ECS discovery

### 5.3.1	General

This clause specifies the requirements for ECS discovery to address the key issue #6 on edge services support across ECSPs in clause 4.6 and to the key issue #10 on support for roaming UEs in clause 4.10.

Editor's note:	How the EDGEAPP architecture works in different roaming scenarios is FFS.

### 5.3.2	Requirements

The following is the architectural requirement to support ECS discovery in the EDGEAPP Rel-18 architecture.

-	The application layer architecture shall provide mechanisms for ECS to discover available ECS which may have suitable EES, to support UE mobility between ECSPs.

## 5.4	Alignment of EDGEAPP and ETSI MEC

### 5.4.1	General

This clause specifies the architecture requirements for alignment of EDGEAPP and ETSI MEC to address the key issue #4.

### 5.4.2	Requirements

The following requirements will serve as the guiding principles for the alignment of EDGEAPP and ETSI MEC architectures.

1.	The scope of 3GPP alignment efforts between EDGEAPP and ETSI MEC shall be limited to architecture enhancements that apply only to EDGEAPP.

NOTE: 3GPP will liaise with ETSI ISG MEC for architecture recommendation (if any).

2.	The architecture enhancements to support alignment shall ensure backwards compatibility with the existing EDGEAPP architecture.

3.	The architecture enhancements shall not consider to align features exclusive to the each of the EDGEAPP and ETSI MEC architectures, i.e. the architecture enhancements shall only focus on the overlapping aspects between the EDGEAPP and ETSI MEC architectures.  The alignment aspects in the present release of this specification includes the following:

a.	alignment of EAS profile (EDGEAPP) and appInfo (ETSI MEC),

b.	alignment of EDGE-3/Mp1 reference points

c.	alignment of EDGE-9/Mp3 reference points

d.	usage of CAPIF between the two architectures

NOTE: The term alignment does not imply that the reference points will be exact equivalents.

Editor's note:	Any additional aspects to be considered for alignment is FFS.

4.	The architecture enhancements shall ensure that EDGEAPP architecture can remain as a standalone or a complete system i.e. EDGEAPP can be deployed independent of the ETSI MEC architecture.

5.	The architecture enhancements shall analyse impacts to other working groups e.g. SA2, SA3 and SA5.

# 6	Enhanced Application Architecture

## 6.1	Option #1: Roaming architecture

This clause describes the architecture for roaming UEs, addressing Key Issue #10.

### 6.1.1	Architecture enhancements

#### 6.1.1.1	Local breakout roaming architecture: Local breakout to access H-ECS

This architecture uses ECSs provided in HPLMN and VPLMN, in which the EEC in the UE obtains services from V-ECS and V-EES. In this architecture, the H-ECS is associated with HPLMN, while the V-ECS and the EDN which the UE accesses is associated with VPLMN. A new reference point EDGE-10 is defined between ECSs. Figure 8.1.2-1 shows the architecture for this model.

NOTE:	H-ECS and V-ECS can be provided by the same ECSP.

Figure 6.1.1.1-1: Local breakout roaming architecture: Local breakout to access H-ECS

#### 6.1.1.2	Home-routed EDGE-4 access to H-ECS

This architecture uses ECSs provided in HPLMN and VPLMN, in which the EEC in the UE obtains services from V-ECS and V-EES. Figure 6.1.1.2-1 shows the architecture for home routed roaming architecture for this model.

Figure 6.1.1.2-1: Home-routed access to H-ECS

In the HR roaming scenario, the roaming architecture is valid if the UE is supported to access the EDN in the VPLMN (i.e., the local access to the EDN of the VPLMN is supported in the HR roaming scenario). The traffic toward the EDN in the VPLMN (i.e., EDGE-1 traffic and application data traffic between AC and EAS) is not home routed to the HPLMN while the traffic between the EEC and H-ECS over is home routed via VPLMN and HPLMN.

Editor's note:	Whether and how to support local access to the EDN in the VPLMN for a HR roaming UE is SA2's responsibility.

### 6.1.2	Identities

None.

### 6.1.3	Cardinality rules

None.

## 6.2	Option #2: Non-roaming architecture

This clause describes the architecture for non-roaming UEs.

### 6.2.1	Architecture enhancements

Compared with the Rel-17 EDGEAPP architecture for the non-roaming scenario, a new reference point EDGE-10 is defined between ECSs. Figure 6.2.1-1 shows the architecture for this model.

NOTE:	ECSs communicating via EDGE-10 may be provided by different ECSPs.

Figure 6.2.1-1: Non-roaming architecture

### 6.2.2	Identities

None.

### 6.2.3	Cardinality rules

None.

## 6.3	Option #3: Edge Notification Server architecture

This clause provides an enhanced application architecture based on Rel-17 architecture by incorporating an Edge Notification Server addressing Key Issue #1, "Enhanced notification service to the EEC".

### 6.3.1	Architecture enhancements

This clause describes the new Edge Notification Server (ENS) functional element and the new interfaces (i.e EDGE-11, EDGE-12 and EDGE-13) needed to enable interactions in between EEC-ENS, EES-ENS and ECS-ENS respectively.

Figure 6.3.1-1 illustrates the reference point representation of the architecture for Edge Enabling Application with the inclusion of the Edge Notification Server and the reference points EDGE-11, EDGE-12 and EDGE-13. The Figure also shows an OEM Push sever which is outside of the PLMN.

NOTE 1: The OEM Push sever and the Push Function in the UE, as shown in Figure 6.3.1-1, are outside the scope of this TR and SA6.

NOTE 2: The details of communication between the ENS and the OEM Push sever is outside the scope of this TR and SA6.

Figure 6.3.1-1: Enhanced architecture using an Edge Notification Server

Editor's Note: Whether ENS should be addressed as a generic function provided as part of SEAL is FFS.

NOTE 3: ENS is an optional feature of the EDGEAPP architecture.

#### 6.3.1.1	Edge Notification Server (ENS)

ENS is the optional central notification server which receives notifications from EES (EDGE-12) and ECS (EDGE-13) and based on the preferred notification delivery method indicated by EEC (over EDGE-11), delivers the notifications to EEC through either a Pull or a Push delivery method.

Functionalities of ENS are:

a)	Enabling EEC to request for a Callback URL to be used in its event subscription creation with EES and ECS;

b)	Enabling EEC to request for an optional Channel URL to receive notifications from the ENS directly (e.g. via Long-polling or WebSocket);

c)	Enabling EEC to request receiving notifications from the ENS indirectly through a preferred Push server (e.g. FCM, APNS, OMA Push);

d)	Setting up a Pull or Push notification channel with EEC based on EEC's preferred notification delivery method (e.g. Long-polling, WebSocket) requested;

e)	Receiving Notifications from EES and ECS at the CallBack URL and passing them onto the EEC either over a notification Channel (e.g. Long-polling, WebSocket) which is setup directly with the EEC or indirectly (i.e. an implicit notification channel) via a Push server (e.g. FCM, APNS, OMA Push);

#### 6.3.1.2	ENS Discovery

For a given EEC, the associated ENS's information (e.g. URI(s), FQDN(s), IP address(es)) and optionally the ENS Provider Identifier are obtained from the ECS as part of the initial provisioning activity (see clause 8.3 in 23.558).

This approach ensures regardless of how many EECs a UE has or how many ENSs are deployed in the network (e.g. one ENS per ECSP or a single ENS by MNO), a given EEC would always receive the appropriate ENS endpoint to communicate and open a notification channel with.

NOTE 1: Depending on the number of EECs (in the UE) and the number of associated ENS(s) in the network (i.e. one ENS per ECSP or a single ENS by MNO), an UE may simultaneously interact with multiple ENSs (one per ECSP) or a single ENS deployed by the MNO.

## 6.4	Option #4: Constrained devices with limited capabilities

### 6.4.1	Architecture enhancements

#### 6.4.1.1	General

This architecture option adds support for certain constrained devices which either don't have enough capabilities to execute its own EEC (e.g., the constrained device may not have a Mobile Termination entity) or do not execute its own EEC to save essential resources such as processing power and battery. Such constrained devices (e.g., terminal equipments as defined in TR 22.944 [21]) will benefit by being able to utilize services of an EEC running on a different UE, using EDGE-5.

#### 6.4.1.2	Architecture

Figure 6.4.1.2-1: Architecture supporting constrained devices

NOTE 1:	The architecture option should utilize existing functionalities specified by SA2 and avoid creating new requirements for system architecture.

NOTE 2:	How the TE communicates with the UE (e.g., Bluetooth, Wi-Fi, Cellular, ProSe etc.) is out of scope of SA6.

### 6.4.2	Identities

None.

### 6.4.3	Cardinality rules

None.

## 6.5	Option #5: Architecture for ACR between EAS and CAS without CES

This clause describes the architecture for enabling interactions between EAS and Cloud Application Server (CAS), addressing Key Issue #11.

### 6.5.1	Architecture enhancements

Figure 6.5.1-1 shows the architecture enabling interactions between EAS and CAS, without Cloud Enabler Server (CES). Compared to the EDGEAPP (Rel-17) architecture, new entity Cloud Application Server is proposed along with the new reference points EDGE-14 (between EES and CAS), EDGE-15 (between ECS and CAS) and EDGE-16 (between 3GPP Core Network and CAS).

Figure 6.5.1-1: Architecture with Cloud Application Server (CAS) and without CES

In this solution, the Cloud Application Server (CAS) interaction with EES is supported via EDGE-14 reference point and CAS is supported by ECS via EDGE-15 reference point. The CAS and EAS interaction is supported as Application Data Traffic, which is out-of-scope of this specification. The CAS interaction with the 3GPP core network happens over EDGE-16 reference point, which is similar to EDGE-7 reference point.

Editor's note:	What functionalities of EDGE-9 and EDGE-6 are to be reused for EDGE-14 and EDGE-15 respectively is FFS.

Since the EAS may have service area restriction, once the UE is moving out of the current edge coverage, to keep service continuity, the application client needs to connect to either another EAS in new EDN or the CAS.

The architecture supports ACR between edge and cloud deployments for the following conditions:

-	Condition 1: For the locations where EDN is not available, the ACR support is based on the failed Service provisioning response (i.e. the non-availability of the EDN at a particular location) from the ECS.

-	Condition 2: When AC profiles are sent in Service provisioning request and particular EAS is not available then EDN non-availability for that EAS is inferred through Service provisioning response.

-	Condition 3: For the locations where EDN is available but the EAS is not available, the ACR support is based on the indication from the EES (in the EAS discovery response) about the non-availability of the EAS at that particular location.

-	Condition 4: EAS and EDN are available but EAS is overloaded or not in a position to serve the EEC/UE due to any reason.

-	Condition 5: For a UE location, when CAS is serving and a suitable EAS is available at the edge, ACR can be initiated for continuing the service delivery via EAS.

### 6.5.2	Identities

None.

### 6.5.3	Cardinality rules

None.

## 6.6	Option #6: Architecture for ACR between EAS and CAS with CES

### 6.6.1	Architecture enhancements

Figure 6.6.1-1: Illustration of application architecture with Edge and Cloud server deployment

In this solution, the Cloud Application Server (CAS) is supported by Cloud Enabler Server (CES) via EDGE-3' reference point and CES is supported by ECS via EDGE-6' reference point. The CES communicates with EES or another CES via EDGE-9' reference point. The EEC utilizes EDGE-1' reference point to communicate with the CES. The CES and CAS interact with 3GPP Core Network via EDGE-7' reference point and EDGE-2' reference point, respectively.

From concept wise, the CAS and CES are servers deployed in a cloud data network and EAS and EES are servers deployed in an edge data network. If certain EAS and EES in the edge data network are capable of supporting more UEs than regular edge server and can serve UE from anywhere (N6 routable), their roles become those of CAS and CES, respectively.

The EDGE prime reference points have the similar functions as the existing EDGE reference points. For instance, CASs are registered via EDGE-3' in CES to enable CES to offer suitable CAS via EDGE-1' to EEC; and CESs are registered via EDGE-6' in ECS to enable ECS to offer suitable CES via EDGE-4 to EEC.

The differences in EDGE prime reference points comparing to existing EDGE reference points are:

-	The CAS does not have service area restriction in CAS profile when registered into CES.

-	The CES does not have service area restriction in CES profile when registered into ECS.

Editor's note:	Further analysis is needed to describe the detailed differences for EDGE prime reference points and the cardinality rules.

### 6.6.2	Identities

None.

### 6.6.3	Cardinality rules

None.

## 6.7	Option #7: Architecture for Common EAS selection with central binding server

### 6.7.1	Architecture enhancements

Figure 6.7.1-1: EDGEAPP architecture enhanced with binding server

The EDGE-X reference point is introduced to support EES interaction with binding server. The EES can store, update and remove the binding information via EDGE-X reference point.

The binding server is deployed in a central cloud and it is a single server maintaining binding information.

Editor's note:	Whether binding server can be deployed in EDN is FFS.

NOTE:	CBS can be co-located with an ECS in a deployment.

### 6.7.2	Identities

None.

### 6.7.3	Cardinality rules

None.

## 6.8	Option #8: Architecture for ACR update in service continuity planning

### 6.8.1	Architecture enhancements

Enhancement of the service continuity planning capability is expected to support update of ACR. As can be seen at the Figure 7.6.1-1, this solution proposes the ACR update capabilities as enhancements after the ACR launch to deal with UE behavior changes. This includes a Detection entity, a Decision Update entity and an ACR update execution entity. These entities can be different based on the scenarios identified in TS 23.558, clause 8.8.2.

Figure 6.8.1-1: high level illustration of proposed service continuity planning enhancements

### 6.8.2	Identities

TBD.

### 6.8.3	Cardinality rules

TBD.

# 7	Solutions

## 7.0	Mapping of solutions to key issues

Table 7.0-1 Mapping of solutions to key issues

|  | KI # 1 | KI # 2 | KI # 3 | KI # 4 | KI # 5 | KI # 6 | KI # 7 | KI # 8 | KI # 9 | KI # 10 | KI # 11 | KI # 12 | KI # 13 | KI # 14 | KI # 15 | KI # 16 | KI # 17 | KI # 18 | KI # 19 | KI # 20 | KI # 21 | KI # 22 | KI # 23 | KI # 24 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #1 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #2 |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #3 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #4 |  |  |  |  |  | X |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #5 |  |  |  |  |  | X |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #6 |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #7 |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #8 |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #9 |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |
| Sol #10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |
| Sol #11 |  | X |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #12 |  |  | X |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #13 |  |  |  |  |  | X |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #14 |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #15 |  |  |  |  |  |  |  | X |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |
| Sol #16 |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #17 |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |
| Sol #18 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |
| Sol #19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |
| Sol #20 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #21 |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #22 |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |
| Sol #23 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |
| Sol #24 |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #25 |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #26 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Sol #27 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |
| Sol #28 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |
| Sol #29 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |
| Sol #30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |
| Sol #31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |
| Sol #32 |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #33 |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #34 |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #35 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |
| Sol #36 |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #37 |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #38 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |

## 7.1	Solution #1: Service provisioning via push notification

### 7.1.1	Architecture enhancements

None.

### 7.1.2	Solution description

#### 7.1.2.1	General

The following solution corresponds to the key issue #1 on enhanced notification service to the EEC in clause 4.1.

In this solution, push notification mechanism is utilized to enhance the service provisioning procedure. It is assumed that at least one push server is available to ECS and the UE has push function supporting the interaction with the push server.

#### 7.1.2.2	Procedure

Pre-conditions:

1.	The UE supports push notification service and the associated push server can be accessed by the ECS.

2.	The address of push server is pre-configured in the Push function.

Figure 7.1.2.2-1: Service provisioning via push notification

1.	The EEC registers with the push function within the UE. The EEC acquires a push token and push server information from the push function.

NOTE:	The push server provides the push function in the UE with a push token, which is delivered to the EEC.

2.	The EEC sends a service provisioning subscribe request to the ECS. The service provisioning request includes push token, push server information (e.g. address) in addition to information elements in clause 8.3.3.3.4 of 3GPP TS 23.558 [2] v17.0.0. The push server address is included as Notification Target Address.

3.	Upon receiving the request, the ECS performs an authorization check as in clause 8.3.3.2.3.2 of 3GPP TS 23.558 [2] and further verify if push notification can be used. If the request is authorized and push server can be used for notification, the ECS creates and stores subscription resource for service provisioning.

4.	If the processing of the request was successful, the ECS responds with a service provisioning subscription response.

5.	If an event occurs at the ECS that satisfies trigger conditions for updating service provisioning of a subscribed EEC and the corresponding subscription is for push notification, the ECS sends push notification request to the push server that is identified by the push server information provided by the EEC in the step 2. The push notification request sent from the ECS to the push server contains the push token, EEC information (e.g. identification or port ID) and service provisioning notification message.

6.	The push server sends the service provisioning notification message to the EEC via the push function, which in turn and delivers the notification message to the corresponding EEC.

### 7.1.3	Solution evaluation

The proposed solution addresses Key Issue #1. The solution is based on Rel-17 service provisioning subscribe procedure and the push notification mechanism supported in a UE. This approach does not require EEC to persistently keep the session with ECS for receiving notification message. Instead, the UE reuses the session kept by the push function and push notification server.

This solution does not introduce impact on Rel-17 architecture.

## 7.2	Solution #2: Traffic filter support for EDGE-3 API addressing application traffic detection

### 7.2.1	Architecture enhancements

None.

### 7.2.2	Solution description

The EAS can provide the domain name to the EES as traffic descriptor and EES can further provision the domain name to the 3GPP CN. The following figure 7.2.2-1 depicts the solution sketch for the Session with QoS create operation with bold text showing the enhancement to the existing procedure in 3GPP TS 23.558 [2], clause 8.6.6.2. Such solution allows the EAS to provide fruitful traffic filters to be applied for the intended session with requested QoS.

Figure 7.2.2-1: Session with QoS API: create operation

In step 1, the EAS requests establishment of a data session between the AC and the EAS with a specific QoS, the EAS sends the domain name as traffic descriptor which also indicates the applicable protocol and matching criteria (e.g. TLS SNI). In step 2a, the EES invokes the PFD management procedure with the 3GPP CN as described in 3GPP TS 23.682 [10] and 3GPP TS 23.502 [8] with an application id that may be derived from the EASID according to local policy. Further the EES provides the same application id for requesting data session with specific QoS in step 2c or step 4.

NOTE:	PFD management is optionally supported in MNO, if EES cannot invoke step 2a, it responds EAS with appropriate error.

If the EAS provides only IP flow description as the traffic filter, the EES can provide the application id (which may be derived from the EASID) to the 3GPP CN when requesting data session with specific QoS and rely on further traffic classification mechanism in MNO to apply specific QoS for the application traffic. In that case, the EES skips the PFD management procedure in step 2a.

Similar approach can be applied for the ACR management event API for EAS to consume EES service.

The EAS can also send other specific filters (e.g. URI) to the EEL allowing a fruitful application traffic detection in the 3GPP CN, such filters will be used by the EES in the PFD management procedure in step 2a.

### 7.2.3	Solution evaluation

This solution address KI#7. It enhances the existing Session with QoS API and ACR management event API in 3GPP TS 23.558 [2] with support for traffic filters more than IP flow description.

## 7.3	Solution #3: Service provisioning triggering via SMS over NAS

### 7.3.1	Architecture enhancements

None.

### 7.3.2	Solution description

#### 7.3.2.1	General

The following solution corresponds to the key issue #1 on enhanced notification service to the EEC.

In this solution, application triggering (device triggering) via SMS over NAS method specified in 3GPP TS 23.501 [5] is utilized to provide the updated EDN configuration information to the EEC.

The EEC indicates to the ECS that SMS over NAS is supported. Then, the ECS checks if the SMS over NAS can be utilized and perform EEC triggering by using SMS over NAS.

#### 7.3.2.2	Procedure

Pre-conditions:

1.	The EEC is able to check if the UE supports the SMS over NAS;

2.	The EEC is able to decode the need to trigger the operation indicated in the SMS; and

3.	The ECS is allowed to use Nnef_Trigger_Delivery service.

Figure 7.3.2.2-1: EEC triggering via SMS over NAS to perform service provisioning

1.	The EEC determines that the UE supports the SMS over NAS by checking if the UE capability support the SMS delivery over NAS and if the UE is allowed by the core network to use SMS delivery over NAS (e.g., whether the UE is indicated by the core network that SMS over NAS is allowed during Registration procedure or UE Configuration Update procedure as specified in TS 23.502 [8]). If the SMS over NAS is supported and allowed in the UE, the EEC sends an indication of SMS over NAS supported to the ECS. This indication may be included in the service provisioning request message and sent to the ECS.

2.	The ECS acknowledges to the EEC by sending a message including whether the EEC triggering via SMS over NAS is available for service provisioning or not. When the EDN configuration information in the ECS is updated, the ECS triggers EEC to initiate service provisioning request by utilizing the SMS over NAS as described in the below figure 7.3.2.2-2.

Figure 7.3.2.2-2: EEC triggering via SMS over NAS to perform service provisioning

1.	The ECS determines to perform EEC triggering via SMS over NAS for service provisioning (e.g. when the EDN configuration information is updated).

2.	The ECS invokes Nnef_Trigger_Delivery service to send a trigger delivery request message to the NEF as described in clause 4.13.2 of 3GPP TS 23.502 [8]. The message includes indication to trigger service provisioning, EEC ID, UE ID, and ECS information (e.g. address).

3.	The NEF delivers the triggering information via MT SMS Delivery procedure as specified in TS 23.502 [8]. The indication to trigger service provisioning, EEC ID, ECS information are included as the triggering payload in the triggering information

4.	The NEF sends Nnef_Trigger_Delivery service response message to the ECS to inform that the ECS-provided information in step 2 is successfully delivered to the SMS-SC as in clause 4.13.2 of 3GPP TS 23.502 [8].

5.	The NEF sends Nnef_Trigger_Delivery Notify message to the ECS to inform that the SMS message is successfully delivered to the UE as in clause 4.13.2 of 3GPP TS 23.502 [8].

6.	The UE receives the SMS message containing the ECS-provided triggering information (indication to trigger service provisioning, EEC ID, ECS information) from the 5GC via Mobile Terminated SMS Delivery as in clause 4.13.3 of 3GPP TS 23.502 [8] and the EEC is triggered to perform service provisioning request toward the ECS identified by the ECS information contained in the SMS message.

7.	The EEC may send a service provisioning request to the ECS identified by the triggering information contained in the SMS.

### 7.3.3	Solution evaluation

The solution addresses Key Issue #1: Enhanced notification service to the EEC.

The solution utilizes application triggering (device triggering) via SMS over NAS method specified in 3GPP TS 23.501 [5]. This solution does not require EEC to persistently keep the session with ECS for receiving the updated EDN configuration information.

This solution has the following impact on the EEC and ECS:

-	EEC supports to perform service provisioning when receiving application triggering SMS containing ECS-provided triggering information;

-	ECS supports application triggering service provided by the core network (e.g. Nnef_Trigger_Delivery service).

This solution does not introduce any impact on Rel-17 architecture.

NOTE:	This solution can be extended to enhance EDGE-1 notification service.

## 7.4	Solution #4: ECS discovery through serving ECS to support edge services across ECSPs

### 7.4.1	Architecture enhancements

For a roaming scenario, Option #1 in the clause 6.1 is the basis for this solution.7.4.2	Solution description

#### 7.4.2.1	General

The following solution corresponds to the key issue #6 on edge services support across ECSPs in clause 4.6 and to the key issue #10 on support for roaming UEs.

In this solution, it is assumed that the ECSP1 and ECSP2 have a service level agreement to share edge services. If the ECS1 cannot discover a suitable EES to serve the UE at the current location (e.g., all the EESs registered on the ECS1 do not cover the given UE location), the ECS1 discovers another ECS2 which may have suitable EES based on UE location and provides it to the requesting EEC or EES.

In a roaming scenario, after a UE selects a PLMN and performs the registration procedure, the EEC gets ECS information to use edge computing service in the VPLMN as follows:

-	The EEC may have been pre-configured or provisioned with V-ECS information.

-	The EEC may try to send a service provisioning request message including service PLMN ID to the H-ECS and retrieves V-ECS information or, if possible, EDN configuration information for VPLMN. The EEC may perform service provisioning procedure with the V-ECS if the H-ECS provides V-ECS information to the EEC as show in figure 7.4.2.2-1. This solution provides a procedure for this aspect.

-	The EEC may try to derive V-ECS address information from the VPLMN identifier. For example, an EEC can combine the VPLMN identifier and an ECSP code (or ECSP ID) assigned to an edge computing service provider (e.g. "ECS <ECSP-ID/CODE>.ECS.MNC <MNC> .mcc <MCC> .3gppNetwork.org ").

In a roaming scenario, if a V-ECS information is provided by 5GC, the EEC shall use the provided information for the subsequent service provisioning request.

NOTE:	In a roaming scenario, current solution provides a procedure to one method for the EEC gets ECS information to use edge computing service in the VPLMN. The precedence among different methods is not the scope of this solution.

#### 7.4.2.2	Procedure

In this solution, when the ECS1 receives the request of EES from the EEC or source EES, the ECS1 discovers another ECS2 which may have suitable EES and responds with the ECS2 information. After that, the EEC or source EES sends the request of EES to the ECS2.

Pre-conditions:

1.	ECSP1 and ECSP2 have a service level agreement to share edge services.

2.	The EEC has a business relationship/subscription to the ECSP1.

3.	The EEC has ECS1 address information and can access to the ECS1 (in roaming scenario, ECS1 and ECS2indicate the H-ECS and the V-ECS, respectively).

Figure 7.4.2.2-1: Solution 4 for edge services support across ECSPs

1.	The ECS1 receives request for obtaining EES information, this procedure may be triggered by following events:

1a.	The EEC sends a service provisioning request to the ECS. It is assumed that the EEC has been pre-configured or has provisioned with the address (e.g. URI) of the ECS1. The request may include the UE location. For roaming scenario, the request may also include serving PLMN ID of the UE hosting the EEC.

1b.	The S-EES (EES1) sends the retrieve EES request to the ECS in order to identify the T-EES which has an EAS available to serve the UE.

2.	If the request does not contain the UE location information, the ECS1 interacts with 3GPP core network to retrieve the UE location. If the ECS1 cannot discover a suitable EES to serve the UE at the received or retrieved UE location based on the received information (e.g., all the EESs registered on the ECS1 do not cover the given UE location), the ECS1 discovers potential ECSs which may have suitable EES based on the information such as the UE location.

The ECS1 may determine a potential ECS information (e.g. address, endpoint or service API information) based on pre-configuration or may discover service API information exposed by that ECS via CAPIF discovery procedure as specified in TS 23.222 [16].

For roaming scenario, if the request does not contain the serving PLMN ID, the ECS1 may interacts with 3GPP core network to retrieve serving PLMN ID. The H-ECS checks if the edge computing service for the EEC can be supported in the VPLMN identified by serving PLMN ID according to the roaming agreement with VPLMN operator for given ECSPs. The ECS1 discovers the potential ECSs, e.g. ECS2 which have suitable EES based on the serving PLMN ID. Optionally, the ECS1 may send a request to ECS2 to verify whether the ECS2 is available. If yes, ECS2 returns the success response to the ECS1. Otherwise, a failure response/code is returned.

The H-ECS can check if the edge computing service for the EEC can be supported in the VPLMN based on the roaming agreement on edge computing services between PLMNs or service agreement between ECSPs. In the roaming scenario, roaming agreement on edge computing service can be addressed by SA2 based on the principles in clause 9.1.

NOTE 1:	The ECS 1 can filter the list of the discovered ECSs based on the information elements provided vis the service provisioning request, ECSP policy configured in ECS, or UE-specific service information. Details of ECSP and UE-specific service information is out of scope.

NOTE 2:	This solution does not exclude other ways for ECS 1 to determine ECS 2. Other solution for determining ECS2 by ECS1 can be utilized in this procedure when the multiple ECSs are available for an EEC.

3. ECS1 sends a response message including the list of ECSs information and failure cause indicating redirection to another ECS (for service provisioning request or retrieve EES request) to the requester of step 1 if the ECS1 cannot discover a EES:

3a.	In response to the request in step 1a, the ECS1 sends the response message to the EEC.

3b.	In response to the request in step 1b, the ECS1 sends the response message to the S-EES.

4.	The EEC or source EES can send the request (e.g. resends a service provisioning request or retrieve EES request) to one of the ECSs received from the ECS1 e.g. the ECS2. Correspondingly, the EEC or source EES receives response from the ECS2.

### 7.4.3	Solution evaluation

Solution 4 solves the problems of KI#10 and KI#6. After discovering another ECS2 which may have suitable EES, the ECS1 sends respond to the EEC or source EES with the ECS2 information. Then the EEC or source EES can send the request to the ECS2 directly to retrieve suitable EES. It is possible that some enhancements on ECS are needed to support ECS discovery.

This solution relies on having sufficient information (e.g., ECSP policy, UE-specific service information, or ECSs information) configured or available in an ECS to determine candidate ECSs in step 2.

Editor's note:	Whether sufficient information (e.g., ECSP policy, or UE-specific service information, or ECSs information) can be configured in an ECS or not is FFS.

The Solution #4 will use any agreed solution for determining ECS2 by ECS1.

## 7.5	Solution #5: ECS enhancement to discover EESs via other ECSs to support edge services across ECSPs

### 7.5.1	Architecture enhancements

A new reference point is required between ECSs, which is described as EDGE-10 in the clause 6.1.1.1.

For a roaming scenario, Option #1 in the clause 6.1 is the basis for this solution.

### 7.5.2	Solution description

#### 7.5.2.1	General

The following solution corresponds to the key issue #6 on edge services support across ECSPs in clause 4.6 and to the key issue #10 on support for roaming UEs in clause 4.10.

In this solution, it is assumed that the ECSP1 and ECSP2 have a service level agreement to share edge services. If the ECS1 cannot discover a suitable EES to serve the UE at the current location (e.g., all the EESs registered on the ECS1 do not cover the given UE location), the ECS1 discovers another ECS2 which may have suitable EES and discovers the EES via the ECS2.

#### 7.5.2.2	Procedure

In this solution, when the ECS1 receives the request of EES from the EEC / source EES, the ECS1 discovers another ECS2 which may have suitable EES and discovers the EES via the ECS2. The ECS1 then provides the EES information to the EEC / source EES in the response.

Pre-conditions:

1.	ECSP1 and ECSP2 have a service level agreement to share edge services.

2.	The EEC has a business relationship/subscription to the ECSP1.

3.	The EEC has ECS1 address information and can access to the ECS1 (in roaming scenario, ECS1 and ECS2 indicate the H-ECS and the V-ECS respectively).

Figure 7.5.2.2-1: Solution 5 for edge services support across ECSPs

1.	The ECS1 receives request for obtaining EES information, this procedure may be triggered by following events:

1a.	The EEC sends a service provisioning request to the ECS. It is assumed that the EEC has been pre-configured or has provisioned with the address (e.g. URI) of the ECS1. The request may include the UE location. For roaming scenario, the request may also include serving PLMN ID of the UE hosting the EEC.

1b.	The S-EES (EES1) sends the Retrieve EES request to the ECS in order to identify the T-EES which has an EAS available to serve the UE.

2.	If the request does not contain the UE location information, the ECS1 interacts with 3GPP core network to retrieve the UE location. If the ECS1 cannot discover a suitable T-EES to serve the UE at the received or retrieved UE location based on the received information (e.g., all the EESs registered on the ECS1 do not cover the given UE location), the ECS1 discovers another ECS2 which may have suitable EES based on the information such as the UE location. The ECS1 may determine the ECS2 information (e.g. address, endpoint or service API information) based on pre-configuration or may discover service API information exposed by that ECS via CAPIF discovery procedure as specified in TS 23.222 [16].

For roaming scenario, if the request does not contain the serving PLMN ID, the ECS1 may interacts with 3GPP core network to retrieve serving PLMN ID. The H-ECS checks if the edge computing service for the EEC can be supported in the VPLMN identified by serving PLMN ID according to the roaming agreement with VPLMN operator for given ECSPs. The ECS1 discovers the ECS2 which have suitable EES based on the serving PLMN ID.

The H-ECS can check if the edge computing service for the EEC can be supported in the VPLMN based on the roaming agreement on edge computing services between PLMNs or service agreement between ECSPs. In the roaming scenario, roaming agreement on edge computing service can be addressed by SA2 based on the principles in clause 9.1.

3.	The ECS1 further discovers a target EES whose service area can cover the UE location via the ECS2. The ECS1 may interact with the ECS2 directly or indirectly via e.g. a Federation manager.

4.	ECS sends the response to the requester of step 1:

4a.	In response to the request in step 1a, ECS1 sends the EES information to the EEC.

4b. In response to the request in step 1b, ECS1 sends the Retrieve EES response to the requesting EES (EES1).

### 7.5.3	Solution evaluation

This solution addresses KI#6 and KI#10. The EEC or the EES gets the requested EES information from ECS2 via ECS1. It is possible that some enhancements on ECS are needed to support service provisioning and EES retrieval request across the ECSPs.

This relies on preconfigured information of ECS2 at ECS1. The solution will use any agreed solution for determining ECS2 by ECS1.

Editor's note:  What is included in the preconfigured information and whether it is enough to determine candidate ECSs in step 2 is FFS.

## 7.6	Solution #6: ACR update in service continuity planning

### 7.6.1	Architecture enhancements

Option #8 in the clause 6.8 is the basis for this solution.

Enhancement of the service continuity planning capability is expected to support update of ACR. As can be seen at the Figure 7.6.1-1, this solution proposes the ACR update capabilities as enhancements after the ACR launch to deal with UE behavior changes. This includes a Detection entity, a Decision Update entity and an ACR update execution entity. These entities can be different based on the scenarios identified in TS 23.558, clause 8.8.2.

Figure 7.6.1-1: high level illustration of proposed service continuity planning enhancements

### 7.6.2	Solution description

The solution addresses the key issue #3 and the open issue on how to deal with scenarios when the ACR needs to be modified, e.g. due to UE mobility.

The solution can have two different solution variants based on the type of service continuity planning update. One possible solution is to modify the ACR service after the launch (described in 7.6.2.1) and another possible solution is the pause of the ACR services for a pre-defined time or till the detection entity decides to resume the ACR (described implicitly in 7.6.2.1 as part of EEC-based modification procedure). ACR pause allows the EEC after the detection of an expected/predicted UE location and/or mobility change with low confidence level, to decide on pausing the ACR and to send to S-EES/T-EES the pause decision and the required time for the pause, or under which criteria the ACR will resume (or a further ACR modification request may follow after some time period if EEC identifies that ACR can continue).

#### 7.6.2.1	ACR modification solution

##### 7.6.2.1.1	EEC-based ACR modification procedure

Pre-conditions:

1.	The ACR has been launched.

Figure 7.6.2.1.1-1: EEC-based ACR modification procedure

1.	The EEC detects a change of the expected UE behaviour. In particular, it may receive from the AC (over EDGE-5) an indication that the expected/predicted UE location and/or mobility changed. Such message can include the current and new expected/predicted location or current and new expected mobility/speed/direction/velocity, and/or an expected change of the UE route/trajectory.

2.	The EEC identifies that one or more ACR updates are needed based on the change of the UE behaviour and decides the type of the ACR update to be an ACR modification or an ACR pause/resume. ACR pause or resume can be a variant of ACR modification and indicates that the ACR needs to be halted for a given time or till further notice (e.g. an ACR resume as ACR update may be decided after some time period if EEC identifies that ACR can continue).

3.	The EEC sends an ACR modification request to the S-EES or T-EES (for EEC executed ACR via T-EES scenario) to indicate an ACR modification and to provide the updated parameters, such as the expected completion time. The request also includes the necessary parameters (e.g. IDs) to indicate the ACR that is requested to be updated. In case of ACR pause, this message indicates the request for an ACR pause and can provide the duration for the pause (time to wait) or under which criteria the ACR is expected to resume. In case of ACR resume, this message indicates the request for an ACR to be resumed after a pause.

4.	S-EES or T-EES determines the ACR to be modified based on the request in step 3. The ACR modification, is executed by the execution entity as described in clause 7.6.2.1.3.

5.  The S-EES (or T-EES for EEC executed ACR via T-EES scenario) sends an ACR modification response to the EEC to notify on the result.

6.	The EEC may optionally provide a notification to the AC (over EDGE-5) to inform on the ACR modification result.

NOTE:	This procedure has impacts on EDGE-5 interface.

##### 7.6.2.1.2	EES-based modification procedure

Pre-conditions:

1.	The ACR has been launched.

Figure 7.6.2.1.2-1: S-EES-based ACR modification procedure

1.	The S-EES detects a change of the expected UE behaviour. In particular, S-EES acting as AF, may receive a UE location report or a monitoring event report from 5GC (assuming that S-EES has subscribed to consume 5GC services like LCS or NEF monitoring events). Such UE location report or monitoring event report may help indicating that the UE is not going to be at the predicted location at the given time and is expected to deviate by the original planning.

2.	The S-EES identifies that one or more ACR updates are needed based on the information on the change of the UE behaviour. The S-EES then decides for each ACR, the type of the ACR update to be an ACR modification and the parameters that need to be updated, such as the expected completion time.

3.	S-EES or T-EES determines the ACR(s) to be modified based on the decision in step 2. For each ACR identified to be updated in step 2, the ACR modification is executed by the execution entity as described in clause 7.6.2.1.3.

4.  The S-EES sends an ACR modification notification to EEC to notify on the result.

5.	The EEC may optionally provide a notification to the AC (over EDGE-5) to inform on the ACR that is modified.

##### 7.6.2.1.3	ACR modification execution procedure

Figure 7.6.2.1.3-1 illustrates the ACR modification execution procedure as used in the procedures in 7.6.2.1.1 and 7.6.2.1.2.

Pre-condition:

1.	An ACR has been launched between EEC, S-EES, S-EAS, T-EES and S-EAS, where S-EES and T-EES can be the same or different; and one of the following preconditions holds:

1.1	The S-EES or T-EES has received an ACR modification request from EEC, e.g. according to the step 3 of the EEC-based ACR modification procedure in solution 6 in clause 7.6.2.1.1; or

1.2	The S-EES has decided an ACR modification, e.g. according to step 3 of EES-based modification procedure in solution 6 in clause 7.6.2.1.2.

Figure 7.6.2.1.3-1: ACR modification execution procedure

1.	If the EAS has subscribed to receive ACR notify for modification, the EES shall notify the EAS about the need to update the ACR. The EES shall include the updated parameters in the notification to the EAS.

Depending on the ACR scenario, the EES can be either of S-EES or T-EES. If it is S-EES, the EAS will be S-EAS. Otherwise if the EES is T-EES, the EAS will be T-EAS.

2.	The EES processes the updated parameters. The EES sends the updated parameters to the remote EES, if the updated parameter can be used by the remote EES to handle the messages from the corresponding EAS, e.g. expected completion time if provided in step 3 of clause 7.6.2.1.1 for the waiting time for the ACR status update.

When EES from step 1 is S-EES the remote EES will be the T-EES. Otherwise if the EES is T-EES, the remote EES will be S-EES.

3.	In response to the message in step 2, the remote EES responds and confirms the reception and acceptance of the updated parameters.

NOTE:  It is assumed that the updated parameters (e.g. updated completion time) do not require the update of any of the EEC context information.

4.	If in step 1 the S-EAS receives the ACR notify for modification, it may initiate ACT update process with the T-EAS. Otherwise if in step 1, T-EAS receives the ACR notify for modification, it may initiate the ACT update process. S-EAS and T-EAS may perform step 4 in an application specific time and manner any time after the notification in step 1.

### 7.6.3	Solution evaluation

This solution provides the procedures to update the parameters of an ACR in service continuity planning. It addresses the third open issue of the KI#3, to handle scenarios when the ACR needs to be modified, e.g. due to UE mobility. The solution does not require enhancement to the existing architecture in Release 17.

## 7.7	Solution #7: EES monitors UE mobility for service continuity planning

### 7.7.1	Architecture enhancements

None.

### 7.7.2	Solution description

#### 7.7.2.1	General

The following solution corresponds to the key issue#3 on enhancements to service continuity planning in clause 4.3

#### 7.7.2.2	Procedure

In this solution, the EES is responsible to identify the ACR type in ACR detection part or receive the ACR type from the EAS and the EEC. When application context transmission is complete, the EAS will send ACR status update message to the EES, once EES detects that UE moves to the predicted/expected location, the EES will notify EEC ACR complete message, then the EEC can be aware of  the compeletion of ACR. With this solution the EAS will not need to monitor the UE mobility and determine when to send the ACR status update message to the EES.

Compared to the procedure specified in 3GPP TS 23.558 clause  8.8.2.5, the following differences are captured below:

2.	The detection entities (EEC, S-EAS, S-EES) detect the ACR may be required and identify the ACID and related ACR type (normal ACR or service continuity planning).

3.	If the EEC or S-EAS detect the ACR event, the EEC or S-EAS should inform S-EES with ACID, ACR type and predicted/expected UE location or EAS service area in the ACR launching procedure.

10. when S-EAS detects application context transmission is complete, the S-EAS sends ACR status update request message to the S-EES with ACID.

11. Once S-EES detects the UE has moved to the predicted/expected UE location or EAS service area, then the S-EES sends ACR complete notify message to the EEC.

Compared to the procedure specified in 3GPP TS 23.558 clause  8.8.2.3, the following differences are captured below:

4.	The EEC performs ACR lauching procedure indicating S-EES with ACR type (normal ACR or service continuity planning) and predicted/expected UE location or EAS service area.7.	When S-EAS detects application context transmission is complete, the S-EAS sends ACR status update request message to the S-EES with ACID.

9. 	Once S-EES detects the UE has moved to the predicted/expected UE location or EAS service area, then the S-EES sends ACR complete notify message to the EEC indicating that UE has moved to the predicted location.

Enhancements to 3GPP TS 23.558 clause 8.8.3.4	ACR launching procedure

Figure 8.8.3.4-1 illustrates the ACR launching procedure by the EEC or the S-EAS. If this procedure is triggered by the EEC, depending on the ACR action indicated in the ACR request, the procedure is used for either ACR initiation or ACR determination. If this procedure is triggered by the S-EAS, the procedure is used for ACR determination.

Pre-condition:

1.	The EEC has been authorized to communicate with the EES as specified in clause 8.11, if the procedure is triggered by the EEC; and

2.	Information related to the S-EES is available with the S-EAS, if the procedure is triggered by the S-EAS.

Figure 8.8.3.4-1: ACR launching procedure

1.	The EEC or the S-EAS sends an ACR request message to the EES in order to start ACR. The ACR request message includes ACR type to indicate whether the ACR is for normal ACR or service continuity planning. The ACR request message includes Predicted/Expected UE location or EAS service area to indicate that the EES should detect whether the UE has moves to the Predicted/Expected UE location or EAS service area or not in ACR clean-up phase. The ACR request message includes ACR action to indicate either ACR initiation request or ACR determination request. If the procedure is triggered by the S-EAS, the ACR request message is only for ACR determination.

An ACR request for ACR initiation:

-	includes an indication of whether the EEC requests the EES to perform EAS notification; and

-	provides information used by EES to perform AF traffic influence as in 3GPP TS 23 501 [2].

An ACR request for ACR determination informs the EES that the need for ACR has been detected at EEC.

2.	The EES checks if the requestor is authorized for this operation. If authorized, the EES processes the request and performs the required operations.

If the request in step 1 is for ACR initiation:

-	the EES may use information provided in the request to apply the AF traffic influence with the N6 routing information of the T-EAS in the 3GPP Core Network (if applicable), as described in 3GPP TS 23.501 [2], clause 5.6.7.1; and

-	if the EAS notification indication is provided in the step 1 request and the EAS has subscribed to receive such notification, the EES shall notify the EAS about the need to start ACR.

If the request in step 1 is for ACR determination, the EES decides to execute ACR as described in clause 8.8.2.5.

3.	The EES responds to the requestor's request with an ACR response message.

Enhancements to 3GPP TS 23.558 clause 8.8.4.4	ACR request

Table 8.8.4.4-1 describes information elements for the ACR request sent from the EEC either to the S-EES or T-EES.

Table 8.8.4.4-1: ACR request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor Identifier | M | Unique identifier of the requestor (i.e. EECID or EASID). |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| UE identifier (NOTE 4) | O | The identifier of the UE (i.e. GPSI). |
| ACR type | M | Indicates whether the ACR is for normal ACR or service continuity planning |
| Predicted/Expected UE location or EAS service area (NOTE 5) | O | The predicted/expected location information of the UE. The UE location is described in clause 7.3.2 or the predicted/expected EAS service area as described in clause 7.3.3.3 |
| Prediction expiration time (NOTE 6) | O | The time the UE reaches the Predicted/Expected UE location or EAS service area at the latest |
| ACR action (NOTE 3) | M | Indicates the ACR action (ACR initiation or ACR determination) |
| ACR initiation data (NOTE 2) | O | ACR initiation IEs to be included in an ACR request message when ACR action indicates it is ACR initiation request. |
| > T-EAS Endpoint | M | Endpoint information (e.g. URI, FQDN, IP 3-tuple) of the T-EAS. |
| > DNAI of the T-EAS | O | DNAI information associated with the T-EAS. |
| > N6 Traffic Routing requirements | O | The N6 traffic routing information and/or routing profile ID corresponding to the T-EAS DNAI. |
| > EAS notification indication | M | Indicates whether to notify the EAS about the need of ACR. |
| > S-EAS endpoint (NOTE 1) | O | Endpoint information of the S-EAS |
| ACR determination data (NOTE 2) | O | ACR determination IEs to be included in an ACR request message when ACR action indicates it is ACR determination request. |
| > S-EAS endpoint | M | Endpoint information of the S-EAS |
| NOTE 1: This IE shall be present if the EAS notification indication indicates that the EAS needs to be informed. NOTE 2: Either ACR initiation or ACR determination shall be included corresponding to the ACR action. NOTE 3: This IE shall indicate ACR determination if the request originates from the S-EAS. NOTE 4: This IE shall be present if the request originates from the EEC. NOTE 5: This IE shall be present if the ACR type indicates the ACR procedure is for service continuity planning. NOTE 6: This IE may be present if the request is from EEC and the ACR type indicates the ACR procedure is for service continuity planning. |  |  |

NOTE:	"Prediction expiration time" and "expected completion time" in solution #6 are the same IE.

NOTE:	The ACR cancellation can be triggered when UE doesn't move to the location monitored by the EES.

### 7.7.3	Solution evaluation

This clause provides an evaluation of the solution.

This solution address KI#3. It enhances the existing service continuity procedure in 3GPP TS 23.558 [2] with support for the EES monitoring UE mobility for service continuity planning. With this solution, the EAS will not need to monitor the UE mobility and determine when to send the ACR status update message to the EES.

## 7.8	Solution #8: EAS Service API enablement using CAPIF

### 7.8.1	Architecture enhancements

None.

### 7.8.2	Solution description

#### 7.8.2.1	General

This solution addresses the Key issue #2: Enablement of Service APIs exposed by EAS as specified in the clause 4.2 by supporting for an EAS to expose its Service APIs towards the other EASs.

As specified in TS 23.558 (Rel-17), the Edge Enabler Layer exposes Service APIs towards the EASs. The exposed Service APIs include the capabilities provided by EES (clause 8.6 of TS 23.558); the capabilities provided by the 3GPP core network (clause 8.7 of TS 23.558); and SEAL service APIs (clause A.4 of TS 23.558).

In this solution, the Edge Enabler Layer also supports for an EAS to expose its Service APIs (i.e., EAS Service APIs) towards the other EASs in order to fulfil the architectural requirements specified in the clause 5.2. This solution exploits CAPIF specified in 3GPP TS 23.222 [16] to support publication/discovery, and change subscription of EAS Service APIs as studied in Sol#15 of TR 23.758 (Rel-17) with the following architectural assumptions within the CAPIF architecture:

-	An EAS may act as an API provider by implementing API provider domain functions (i.e., API exposing function, API publishing function, and API management function)

-	An EAS may act as an API invoker

-	An EES may act as a CAPIF provider by implementing CAPIF core function (CCF)

Note:

-	EES provides support for the logging and audit of EAS API invocation as described in 3GPP TS 23.222[16], clause 8.19 and clause 8.22.

-	EES control the service API access as specified in clause 8.12 of 3GPP TS 23.222 [16].

Based on the architectural assumption above, the essential operations regarding EAS Service APIs complying with CAPIF are as follows:

-	An EAS (acting as API provider) may publish its EAS Service APIs to EES (acting as CAPIF provider) and EES may further publish the EAS service APIs to remote EES via CCF interconnection.

-	An EAS (acting as API invoker) may discover EAS Service APIs from EES (acting as CAPIF provider) and EAS Service APIs from remote EES via CCF interconnection

-	An EAS (acting as API invoker) may subscribe to be notified of dynamic information or availability of EAS Service APIs from EES (acting as CAPIF provider)

-	As EAS1(acting as API invoker) discovers EAS2 (acting as API provider) from EES, it can invoke the Service APIs directly from EAS2

#### 7.8.2.2	CAPIF operations in Edge Enabler Layer

The Figure 7.8.2.2-1 depicts the essential operational steps for EAS Service API enablement using the CAPIF operations as shown in Annex A of TS 23.222 [16].

Pre-conditions:

1.	The EAS #A-1 and EAS #A-2 have completed the EAS registration with the EES #A.

Figure 7.8.2.2-1: CAPIF operations in Edge Enabler Layer for EAS Service API enablement

1.	The EAS #A-2 (AMF) registers its API provider domain functions to the EES #A (CCF) via CAPIF-5.

2.	The EAS #A-2 (APF) publishes its exposing Service API(s) to the EES #A (CCF) via CAPIF-4.

3.	The EAS #A-1 (API invoker) performs onboarding process with the EES #A (CCF) via CAPIF-1.

4.	The EAS #A-1 (API invoker) discovers from the EES #A (CCF) a Service API required to run via CAPIF-1.

5.	The EAS #A-1 (API invoker) invokes the Service API provided by EAS #A-2 (AEF) via CAPIF-2 as discovered from the EES #A (CCF).

6. 	The EAS #A-1 (API invoker) subscribes to notifications of any updates of target Service APIs on the EES #A (CCF) via CAPIF-1.

7.	The EES #A (CCF) and EES #B (CCF) inter-operate with each other via CAPIF-6 for interconnection operations for publication and discovery of Service APIs managed by each EES.

The Figure 7.8.2.2-2 depicts the distributed deployment of EESs (CCFs) as shown in clause 7.4 of TS 23.222 [16].

Figure 7.8.2.2-2 Distributed EES (CCF) deployment for API invocation across EES

The essential operations regarding EES deployment and across EES API invocation complying with CAPIF are as follows

-	In the distributed deployment, EES#A (acting as CCF#A) and EES#B (acting as CCF#B) interact with CCF#C via CAPIF-6 reference point. The CCF#C assumes the role of a centralized repository of the service APIs in the PLMN trust domain.(e.g., Service APIs exposed by EAS #A-2 and EAS #B in the Figure)

-	The EES#A and EES#B publishes the service API(which are published by EAS #A-2 and EAS #B respectively) provided by its connected API exposing function(s) to CCF#C, and obtains the service API information provided by other EES from centralized CCF#C

-	EAS#A-1 (acting as API invoker) is able to discover and invoke the service APIs provided by EAS#B connected to CCF#B

#### 7.8.2.3	Service KPIs in CAPIF for EAS Service APIs

In TS 23.558, "Service KPI" IEs are specified to provide information about service characteristics provided by EASs; or required by ACs. This is used for discovery or provisioning of EASs which meet the Service KPIs required by ACs.

In the similar manner, Service KPIs of EASs (as API Providers) need to be specified in CAPIF to be used for discovery or provisioning of EAS Service APIs which meet the Service KPIs required by EASs (as API Invokers).

The proposed IEs in CAPIF as specified in 3GPP TS 23.222 [16] related to Service KPIs can be summarized as follows:

1)	Service KPIs provided by EAS as API Provider

a.	Service API publish request

i.	Service API information

-	Service KPI (new IE): information about service characteristics provided by the Service API; can be mapped to EAS Service KPIs in EAS Profile [TS 23.558] of the EAS providing the Service API

2)	Service KPIs required by EAS as API Invoker

a.	Onboard API invoker response

i	Service API information

-	Service KPI per API (new IE): information about service characteristics provided by the Service API which is allowed to access

b.	Service API discover request

i	Query information

-	Service KPI (new IE): information about service characteristics as a criterion for discovering matching Service APIs required by the API invoker

c.	Service API discover response

i	Service API information

-	Service KPI per API (new IE): information about service characteristics provided by the Service API corresponding to the discovery request

3)	Service KPIs for CAPIF interconnection between EESs

a.	Interconnection API publish request

i.	Service API information

-	Service KPI (new IE): information about service characteristics provided by the Service API which is published across CCFs (i.e., implemented in EESs)

b.	Interconnection service API discover request

i	Query information

-	Service KPI (new IE): information about service characteristics as a criterion for discovering matching Service APIs across CCFs (i.e., implemented in EESs)

The relevant information elements of CAPIF are listed as follows. The highlighted with bold text is proposed to add for supporting the Service KPIs.

Table 7.8.2.3-1: Service API publish request (Table 8.3.2.1-1 of TS 23.222 [16])

| Information element | Status | Description |
| --- | --- | --- |
| API publisher information | M | The information of the API publisher may include identity, authentication and authorization information |
| Service API information | M | The service API information includes the service API name, service API type, communication type, description, Serving Area Information (optional), AEF location (optional), interface details (e.g. IP address, port number, URI), protocols, version numbers, and data format, (new) Service KPI. |
| Shareable information | O (see NOTE) | Indicates whether the service API or the service API category can be published to other CCFs. And if sharing, a list of CAPIF provider domain information where the service API or the service API category can be published is contained. |
| NOTE: If the shareable information is not present, the service API is not allowed to be shared. |  |  |

Table 7.8.2.3-2: Onboard API invoker response (Table 8.1.2.2-1 of TS 23.222 [16])

| Information element | Status | Description |
| --- | --- | --- |
| Onboarding status | M | The result of onboarding request i.e., success indication is included if the API invoker is granted permission otherwise failure. |
| Enrolled information | O (see NOTE 1) | Information from the provisioned API invoker profile which may include information to allow the API invoker to be authenticated and to obtain authorization for service APIs |
| Service API information | O (see NOTE 2) | The service API information includes the service API name, service API type, communication type, description, Serving Area Information (optional), AEF location (optional), interface details (e.g. IP address, port number, URI), protocols, version numbers, and data format, (new) Service KPI. |
| Reason | O (see NOTE 3) | This element indicates the reason when onboarding status is failure. |
| NOTE 1: Information element shall be present when onboarding status is successful. NOTE 2: Information element may be present when onboarding status is successful. NOTE 3: Information element shall be present when onboarding status is failure. |  |  |

Table 7.8.2.3-3: Service API discover request (Table 8.7.2.1-1 of TS 23.222 [16])

| Information element | Status | Description |
| --- | --- | --- |
| API invoker identity information | M | Identity information of the API invoker discovering service APIs |
| Query information | M | Criteria for discovering matching service APIs (e.g. service API type, Serving Area Information (optional), preferred AEF location (optional), interfaces, protocols, (new) Service KPI) (see NOTE) |
| NOTE: It should be possible to discover all the service APIs. |  |  |

Table 7.8.2.3-4: Service API discover response (Table 8.7.2.2-1 of TS 23.222 [16])

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of the discovery of the service API information |
| Service API information (see NOTE 2) | O (see NOTE 1) | List of service APIs corresponding to the request, including API description such as service API name, service API type, Serving Area Information (optional), interface details (e.g. IP address, port number, URI), protocols, version, data format, (new) Service KPI |
| CAPIF core function identity information | O (see NOTE 1) | Indicates the CAPIF core function serving the service API category provided in the query criteria |
| NOTE 1: The service API information or the CAPIF core function identity information or both shall be present if the Result information element indicates that the service API discover operation is successful. Otherwise both shall not be present. NOTE 2: If topology hiding is enabled for the service API, the interface details shall be the interface details of AEF acting as service communication entry point for the service API. |  |  |

Table 7.8.2.3-5: Interconnection API publish request (Table 8.25.2.1-1 of TS 23.222 [16])

| Information element | Status | Description |
| --- | --- | --- |
| CCF information | M | The information of the CAPIF core function which publishes APIs, may include identity, authentication and authorization information |
| Service API information | O (see NOTE 1) | The service API information includes the service API name, service API type, communication type, description, Serving Area Information (optional), AEF location (optional), interface details (e.g. IP address, port number, URI), protocols, version numbers, and data format, (new) Service KPI. |
| Service API category | O (see NOTE 1) | The category of the service APIs to be published, (e.g. V2X, IoT) |
| Shareable information | O (see NOTE 2) | Indicates whether the service API or the service API category can be published to other CCFs. And if sharing, a list of CAPIF provider domain information where the service API or the service API category can be published is contained. |
| NOTE 1: At least one of the Service API information or Service API category shall be present. NOTE 2: If the shareable information is not present, the service API is not allowed to be shared. There is one and only one CAPIF provider domain information sharable via the CAPIF-6e interface. |  |  |

Table 7.8.2.3-6: Interconnection service API discover request (Table 8.25.2.3-1 of TS 23.222 [16])

| Information element | Status | Description |
| --- | --- | --- |
| CAPIF core function identity information | M | Identity information of the CAPIF core function discovering service APIs |
| Query information | M | Criteria for discovering matching service APIs or CAPIF core function (e.g. service API type, Serving Area Information (optional), preferred AEF location (optional), interfaces, protocols, service API category, (new) Service KPI) (see NOTE) |
| NOTE: It should be possible to discover all the service APIs. |  |  |

The EAS Service KPIs is defined as below

Table 7.8.2.3-7: EAS Service KPIs

| Information element | Status | Description |
| --- | --- | --- |
| Maximum Request rate | O | Maximum request rate from the API Invoker supported by the server. |
| Maximum Response time | O | The maximum response time advertised for the API Invoker's service requests. |
| Availability | O | Advertised percentage of time the server is available for the API Invoker's use. |
| Available Compute | O | The maximum compute resource available for the API Invoker. |
| Available Graphical Compute | O | The maximum graphical compute resource available for the API Invoker. |
| Available Memory | O | The maximum memory resource available for the API Invoker. |
| Available Storage | O | The maximum storage resource available for the API Invoker. |
| Connection Bandwidth | O | The connection bandwidth in Kbit/s advertised for the API Invoker's use. |
| NOTE: If API invoker is on UE then the maximum response time includes the round-trip time of the request and response packet, the processing time at the server and the time required by the server to consume 3GPP Core Network capabilities, if any. |  |  |

In TS 23.222 [16], the Service KPIs can be specified as a part of Query information and Service API information in abstract, so that the Service KPI IE can be implemented as one or more new attributes of ServiceAPIDescription data type in TS 29.222 (but it's up to the CT3's work).

### 7.8.3	Solution evaluation

This solution allows for an EAS to expose its Service APIs towards the other EASs. It exploits CAPIF as specified in 3GPP TS 23.222 [16] for publication/discovery, and change subscription of EAS Service APIs.

This solution addresses the key issue #2: enablement of service APIs exposed by EAS as specified in the clause 4.2; and fulfils the architectural requirements for enablement of service APIs exposed by EAS as specified in the clause 5.2.

This solution relies on the EDGEAPP architecture as specified in TS 23.558 [2] with extended capabilities of EAS and EES for publication/discovery of EAS Service APIs but there is no impact to the APIs and procedures of EDGAPP since the extended capabilities can be realized using CAPIF.

This solution relies on the CAPIF as specified in TS 23.222 [16] with some updates on the information elements of the CAPIF APIs for publish/discovery.

## 7.9	Solution #9: Application traffic influence trigger from EAS

### 7.9.1	Architecture enhancements

None.

### 7.9.2	Solution description

#### 7.9.2.1	General

This solution addresses key issue 14. An EAS can explicitly request EES including necessary information to influence the EAS traffic from UE(s). Then the EES can trigger the AF request to influence traffic routing towards the 3GPP CN for one or more UE(s) accessing the EAS.

#### 7.9.2.2	Procedure

Figure 7.9.2.2:  AF influence the traffic for EAS

1.	The EAS sends an EAS traffic influence request, the request consists of necessary parameters such as N6 Traffic Routing requirements, the Target UE Identifier(s), Temporal Validity Condition.

2.	The EES performs an authorization check to verify whether the EAS has the authorization to request application traffic influence.

3.	Upon successful authorization, the EES sends the traffic influence request to 5GC to influence the traffic for EAS as described in 3GPP TS 23.501, clause 5.6.7.1.

4.	The EES sends the EAS traffic influence response.

Editor's note: The information flow details corresponding to step 1 and 4 is FFS.

### 7.9.3	Solution evaluation

This clause provides an evaluation of the solution.

## 7.10	Solution #10: low power mode support

### 7.10.1	Architecture enhancements

None.

### 7.10.2	Solution description

#### 7.10.2.1	General

The following solution corresponds to the key issue #15 on supporting of constrained devices for Edge.

For the constrained UEs, there are two aspects should be considered to reduce the power consumption, one is help UE turn into idle mode or DRX mode when there is no traffic transmitting between UE and network, another one is help to reduce the traffic data rate when UE is connected to the network with transmitting packets.

EEC can get the knowledge about the constrained UE. When the applications are running with high power consumption, or the UE decides that the battery level is low and should start power saving, the EEC should send messages to EES with carrying such indication for low-power consumption requirement and specific application identifier.

When EES gets low-power consumption indication from EEC, the EES should trigger two methods to satisfy the constrained UE requirement.

Method1: EES should send the message to corresponding EAS through EDGE-3 that the data rate should be reduced, so that the downlink data will be reduced and UE do not need to process high-bandwidth traffic. How to balance the low-power consumption requirement and the UE experience is based on the EAS implementation.

Method2: EES should send the UE's low-power consumption indication to 3GPP core network. The 3GPP system may trigger some specific policies or treatment for such applications and UEs e.g. changing the AM policies e.g. update UE-AMBR, UE- slice MBR, RFSP index and/or service area restriction, for those UEs as specified in TS 23.502 clause 4.15.6.9, and changing the SM policies e.g. downgrade the QoS as specified in TS 23.502 clause 4.15.6.6.

Editor's note: It is FFS whether AM policy change can satisfy the need for all applications in the UE.

#### 7.10.2.2	Procedure

Pre-condition:

1.	The AC-EAS cannot handle the negotiation at the application layer.

Figure 7.10.2.2-1: low power mode support

1.	The EEC sends low-power consumption indication to EES with UE Identifier, EEC ID, AC ID, low-power consumption indicator, which indicating a constrained UE, and triggers the low power consumption enhancement.

2a.	When EES gets the low-power consumption indication, the EES may send low power indication to EAS to request corresponding EAS to reduce the downlink packets, which can help constrained UE process the application with constrained power consumption. The request could provide the UE Identifier(s).

2b. When EES gets the low-power consumption indication, the EES may request 3GPP core network to update the UE policy or QoS policy to support constrained UE, e.g. EES could act as an AF to set up a session with required QoS procedure as specified in TS 23.502 clause 4.15.6.6.

Editor's note: How much benefit does QoS downgrade have for energy saving is FFS.

3a. The EAS responds to EES to indicate the low-power processing is successful or failure.

3b. the 3GPP core network responds to EES with successful or failure.

4. EES response to EEC.

### 7.10.3	Solution evaluation

This clause provides an evaluation of the solution.

## 7.11	Solution #11: A deployment option for alignment with ETSI MEC using CAPIF

### 7.11.1	Architecture enhancements

None.

### 7.11.2	Solution description

#### 7.11.2.1	General

This solution addresses "KI#5: Alignment of EDGEAPP and ETSI MEC" in alignment with ETSI MEC using CAPIF for exposing and invoking Service APIs as well as "KI#2: Enablement of Service APIs exposed by EAS" for exposing EAS Service APIs to ETSI MEC entities. It specifies a CAPIF deployment option for:

1.	allowing EASs to invoke MEC services, and

2.	exposing Service APIs provided by EES or EAS (as described in KI#2) to MEC Applications

The following architecture is proposed to use CAPIF for alignment with ETSI MEC in terms of exposing EAS|EES Service APIs and invoking MEC Services as a deployment option.

Based on the proposed deployment option, the alignment of EDGEAPP and ETSI MEC could be done by mapping or complementing their own APIs to CAPIF APIs, especially for invoking/exposing Service APIs and MEC Services.

Note that Figure 7.11.2.1-1 below is to illustrate what EDGEAPP and ETSI MEC could look like when they exploit CAPIF for invoking and exposing their APIs. There is no such architecture defined in EDGEAPP or ETSI MEC as shown in Figure 7.11.2.1-1.

Figure 7.11.2.1-1: A deployment option for alignment with ETSI MEC using CAPIF

The proposed deployment option can be realized with the following implementations:

1)	3GPP EDGEAPP

a.	EES may implement CAPIF Core Function to manage the Service APIs exposed by EAS and EES;

b.	EES may implement CAPIF Core Function to interconnect with CCF of ETSI MEC via CAPIF-6

c.	EAS may implement CAPIF API Provider functions in order to expose EAS Service APIs to EDGEAPP EASs or ETSI MEC Applications

d.	EAS may implement CAPIF API Invoker function to invoke EAS Service APIs or ETSI MEC Services

e.	EAS may invoke EES Service APIs via EDGE-3

f.	EES may implement CAPIF API Provider functions in order to expose EES Service APIs (e.g. UE location API, AC information exposure API, etc.) to API invokers (e.g. EASs, ETSI MEC Applications)

-	i.e. EES Service APIs (as well as EAS Service APIs) need to be published to CAPIF Core Function

2)	ETSI MEC (out of the SA6 scope)

a.	ETSI MEC (e.g. MEP) may implement CAPIF Core Function to manage the MEC Services exposed by MEP and MEC App.

b.	ETSI MEC (e.g. MEP) may implement CAPIF Core Function to interconnect with CCF of EDGEAPP EES via CAPIF-6

c.	ETSI MEC (e.g. MEP, MEC App.) may implement CAPIF API Provider functions in order to enable EDGEAPP EASs to invoke MEC Services via CAPIF-2

d.	ETSI MEC (e.g. MEC App.) may implement CAPIF API Invoker function to invoke EDGEAPP EAS|EES Service APIs

NOTE:	Refer to Annex B.2 of ETSI GS MEC 011 [14] for mapping MEC service management API to CAPIF APIs.

#### 7.11.2.2	Procedure

The procedures for API publication, API discovery, API invocation, and CCF interconnection are as specified in TS 23.222 [16].

### 7.11.3	Solution evaluation

This solution provides a deployment option for alignment with ETSI MEC using CAPIF.

This solution addresses KI#5: Alignment of EDGEAPP and ETSI MEC as specified in the clause 5.5; and KI#2: Enablement of Service APIs exposed by EAS as specified in the clause 5.2.

This solution relies on the EDGEAPP architecture as specified in TS 23.558 [2] with extended roles of EAS and EES as CAPIF entities and extended roles of ETSI MEC entities like MEP as CAPIF entities to support the alignment of discovery and invocation of EES|EAS Service APIs and MEC Services by ETSI MEC and EDGEAPP entities, respectively.

This solution also relies on the CAPIF as specified in TS 23.222 [16].

## 7.12	Solution #12: Service continuity planning permission

### 7.12.1	Architecture enhancements

None.

This solution provides a deployment option for alignment with ETSI MEC using CAPIF.

This solution addresses KI#5: Alignment of EDGEAPP and ETSI MEC as specified in the clause 5.5; and KI#2: Enablement of Service APIs exposed by EAS as specified in the clause 5.2.

This solution relies on the EDGEAPP architecture as specified in TS 23.558 [2] with extended roles of EAS and EES as CAPIF entities and extended roles of ETSI MEC entities like MEP as CAPIF entities to support the alignment of discovery and invocation of EES|EAS Service APIs and MEC Services by ETSI MEC and EDGEAPP entities, respectively.

This solution also relies on the CAPIF as specified in TS 23.222 [16].

### 7.12.2	Solution description

#### 7.12.2.1	General

This solution addresses the key issue #3 on enhancements to service continuity planning and the key issue #12 on EEL service differentiation.

In this solution, the EEC provides its capability to perform service continuity planning and the EES can provide information about allowing the service continuity planning.

#### 7.12.2.2	Procedure

Pre-conditions:

1.	EEC knows whether the planned or predicted UE mobility behaviour is available.

2.	The EES is locally configured with the UE-specific service information (as specified in the NOTE 2 in clause 8.5.2.2 of TS 23.558) and the ECSP policy indicating whether the service continuity planning is allowed or not for each EAS(s).

NOTE:	Details of the ECSP policy and how to configure such ECSP policy into the EES is not the scope of this solution

Figure 7.12.2.2-1: Service continuity planning permission during EEC Registration procedure

1.	The EEC provides EEC Service Continuity Support or indication to request service continuity planning permission when sending EEC Registration request. The request message may contain the capability of EEC to detect and provide information about planned or predicted UE mobility behaviour (e.g. including whether the UE moves to the expected/predicted location or not).

2.	If the EEC Registration request message includes EEC Service Continuity Support or indication to request service continuity planning permission, the EES verifies if the service continuity planning is allowed or not for the EEC based on the ECSP policy or the request from the EAS provider. If the EEC indicates to the EES in step1 that it cannot detect information about predicted UE mobility behaviour, then the EES monitors predicted UE mobility (e.g. including whether the UE moves to the expected/predicted location or not for service continuity planning.).

If AC profile(s) is provided in step 1, the EES may verify whether the service continuity planning is allowed or not for each EAS(s) matched with the AC profile(s) based on the ECSP policy or the request from the EAS provider.

3.	The EES sends EEC Registration response including indication on whether service continuity planning is allowed or not.

If the EEC receives the indication that service continuity planning is allowed in step 3, and after the available ACR scenarios are determined for the EEC, the EEC detects or acquires the planned or predicted UE mobility behaviour according to the determined ACR scenarios. If the EES verifies and indicates the service continuity planning permission per application to the EEC in step 2-3, the EEC performs the required detection or acquisition of the planned or predicted UE mobility behaviour for the EASs allowed for service continuity planning according to the determined ACR scenarios.

If the EEC is indicated that service continuity planning is not allowed, the EEC does not issue the application context relocation request for service continuity planning.

NOTE:	If the EEC is indicated that service continuity planning is not allowed, the EEC does not perform detecting or acquiring the planned or predicted UE mobility behaviour for the purpose of service continuity planning.

### 7.12.3	Solution evaluation

This solution addresses the key issue #3 on enhancements to service continuity planning and the key issue #12 on EEL service differentiation. The proposed solution enables ECSP(s) to allow the service continuity planning selectively for UEs or for applications. For example, the service continuity will be enabled only if the resources are sufficient and if the UE (e.g. of a premium user) is allowed for the predicted application context relocation. In this regard, the solution #13 proposes potential impact on information to communicate within EEL in order to enable service differentiation of the service continuity planning. Specifically, the solution introduces the following impacts on the EEC and the EES:

-	The EEC supports to provide indication to request service continuity planning permission and its capability to perform service continuity planning; and

-	The EES supports to verify if the service continuity planning is allowed and provide information about the permission of the service continuity planning to the EEC.

Editor's note:	Potential backward compatibility issue should be evaluated for this solution

## 7.13	Solution #13: Update ECS configuration information

### 7.13.1	Architecture enhancements

None.

### 7.13.2	Solution description

This solution addresses key issue #6 on edge services support across ECSPs and key issue #10 on support for roaming UEs.

This solution proposes to include supported PLMN ID(s) in the ECS configuration information as described in the following table:

Table 7.13.2-1: ECS configuration information provided by ECS to 5GC

| Information element | Status | Description |
| --- | --- | --- |
| … | … | … |
| Supported PLMN ID(s) | O | The identifier of the PLMNs of which EDN configuration information can be provided by the ECS as spatial validity condition. |

NOTE 1:	Except in LBO roaming scenario, when the ECS configuration in Table 7.13.2-1 is successfully stored in the UDM, the H-SMF can provide ECS Configuration Information including a VPLMN ID. In this case, UE can use the ECS Configuration Information to get EDN configuration information of the VPLMN only when the UE is registered via a cell of the VPLMN.

The above ECS configuration information may be provisioned to the 5GC procedure (see 3GPP TS 23.502 [8]) or configured in an ECS so that it can be used to select proper other ECS in solution #4 of clause 7.4.

NOTE 2:	How to provision the updated ECS configuration information via the 5GC is SA2's responsibility.

Editor's Note:	It is FFS whether additional IE such as supported ECSP ID(s) is included in the ECS configuration information.

Editor's Note:	It is FFS whether the Supported PLMN ID(s) needs to be provided to the EEC.

### 7.13.3	Solution evaluation

This solution addresses key issue #10 on support for roaming UEs.

This solution proposes to include additional information in the ECS configuration information in order to provide the 5G core network and the EEC with PLMN ID(s), which can be used to select ECS to be used in scenarios of roaming.

This solution has no architectural enhancements.

## 7.14	Solution #14: V-ECS Discovery via the H-ECS

### 7.14.1	Architecture enhancements

The EDGE-10 reference point, which is shown in clause 6.1.1.1, may be used between the V-ECS and H-ECS.

### 7.14.2	Solution description

#### 7.14.2.1	General

This solution addresses aspects of Key Issue #10. Specifically, the solution explains how the EEC in the roaming UE knows the availability of ECS(s) and/or EES(s) and discovers them in the VPLMN.

The principle of this solution is that, when roaming, the H-ECS provides information to the UE so that the EEC can communicate with an ECS in the VPLMN (i.e. a V-ECS). The UE can then use a home routed PDU Session to communicate with an H-ECS and an LBO PDU Session to communicate with a V-ECS. In this solution, the home network does not need to provide V-EES information to the EEC. The EEC can discover V-EES information by interacting directly with the V-ECS.

As explained in 3GPP TS 23.501 [5]: "The HPLMN can control via subscription data per DNN and per S-NSSAI whether a PDU Session is to be set-up in HR or in LBO mode." This solution explains how the H-ECS can provide information to the UE that can be used to establish an LBO PDU Session that is used to reach a V-ECS.

As one option, the H-ECS can provide an FQDN or an IP Address of a V-ECS to the EEC. When the UE accesses the provided FQDN or IP Address, URSP rules may steer the UE to use a DNN/S-NSSAI combination that can be used to reach the V-ECS (e.g. an LBO Session).

As a second option, the H-ECS can provide a DNN / S-NSSAI combination to the EEC. In this case, the DNN / S-NSSAI combination may be used to send a PDU Session Establishment Request that will result in an LBO PDU Session. The SMF in the VPLMN may then send ECS Address Configuration Information to the UE as described in 3GPP TS 23.548 [19]. The EEC may then use the ECS FQDN or IP Address from the ECS Address Configuration Information to reach the V-ECS.

#### 7.14.2.2	Procedure

This procedure presents a high-level overview of Solution #14.

Figure 7.14.2.2-1: V-ECS Discovery via the H-ECS

1.	The EEC sends a Service Provisioning Request to an ECS in the home network via a home routed PDU session. As described in 3GPP TS 23.558 [2], clause 8.3.3.3.7, the Service Provisioning Request may include the identity of the PLMN that the UE is currently registered to.

2.	If the Service Provisioning Request from the EEC did not include a PLMN ID, then the ECS may obtain the PLMN ID by invoking the NEF's monitoring event API with the monitoring type set to ROAMING_STATUS and the plmnIndication set to TRUE as described in 3GPP TS 29.522 [17] and 3GPP TS 29.122 [18]. The ECS receives a response from the NEF. The response indicates if the UE is roaming and the identity of the UE's serving PLMN.

3.	If the H-ECS is already provisioned with ECS Discovery Information that can be sent to the UE (e.g. via OAM), then steps 3 and 4 may be skipped. In this step, the H-ECS may send a V-ECS Information Request to a V-ECS of the PLMN that the UE is currently registered in order to obtain the information that should be sent to the UE so that the UE can establish an LBO PDU Session in the VPLMN. The V-ECS Information Request can include the UE's location information.

The H-ECS can use the PLMN ID to determine the V-ECS to contact. For example, this can be based on a DNS lookup or the addresses of V-ECSs may have been pre-configured in the H-ECS (e.g. via OAM). If the VPLMN hosts multiple V-ECSs, then which V-ECS is resolved can be based on configuration.

4.	The V-ECS responds to the H-ECS with a V-ECS Information Response. The V-ECS Information Response can include an DNN (O), and S-NSSAI (O) that can be used by the UE to establish an LBO PDU Session in the VPLMN. The V-ECS can use the UE's location to determine what DNN (O), and S-NSSAI (O) should be sent to the UE in order to cause the UE to establish an LBO PDU Session with a suitable DNN/S-NSSAI combination.

NOTE:	The information that is used by the H-ECS to contact the V-ECS can be different than the information that is used by the EEC to contact a V-ECS. As shown in Figure 7.14.2.2-1, the ECS that is contacted by the UE may be different.
Example 1:	The UE only requires a DNN/S-NSSAI combination to establish an LBO PDU Session and get provisioned with a V-ECS FQDN during PDU Session Establishment.

4.	The ECS sends a Service Provisioning Response. The Service Provisioning Response may include a new information element called ECS Discovery Information. The ECS Discovery Information may include ECS Contact Information (O), ECS Provider ID (O), DNN (O), and S-NSSAI (O).

6.	The UE establishes an LBO PDU Session that will be used to communicate with a V-ECS and/or to obtain ECS Contact Information from the VPLMN.

The ECS Discovery Information from step 5 may include only ECS Contact Information (i.e. an FQDN of an ECS in the VPLMN). In this case, URSP rules may steer the UE to use a DNN/S-NSSAI combination that can be used to reach the ECS (e.g. an LBO Session).

The ECS Discovery Information from step 5 may include a DNN / S-NSSAI combination. In this case, the combination may be used to send a PDU Session Establishment Request that will result in an LBO session. The SMF in the VPLMN may then send ECS Address Configuration Information to the UE as described in 3GPP TS 23.548 [19].

7.	The EEC sends a Service Provisioning Request to an ECS in the visited network. The ECS that is contacted in this step (i.e. V-ECS-2) may be different than the ECS that was contacted in steps 3 and 4 (i.e. V-ECS-1). The ECS address was obtained in step 5 or step 6.

8.	The EEC receives a Service Provisioning Response via the LBO PDU session that was established in step 6. As described in step 5, the ECS service provisioning response may include ECS Contact Information for other ECS(s) in the VPLMN. For example, the ECS that was contacted in step 7 may be a "primary" ECS of the VPLMN and the "primary" ECS of the VPLMN may provide the UE with ECS Contact Information for other ECS(s) in this step.

### 7.14.3	Solution evaluation

Solution #4 and this solution share the principle that the home network can provide the UE with information that is used to contact a V-ECS. In solution #4 the V-ECS information may be an address, endpoint or service API information. In this solution, the V-ECS information can include a DNN (O) and/or S-NSSAI (O). The DNN / S-NSSAI can be used to establish an LBO PDU Session. Once the UE establishes an LBO PDU Session, Rel-17 procedures can be used to discover the V-ECS address. This solution and Solution #5 both include an option where EDGE-10 is used between the H-ECS and V-ECS to obtain the V-ECS information that can be sent to the UE. For example, the V-ECS may provide the H-ECS with a DNN/S-NSSAI combination that can be used by the UE to establish an LBO PDU Session. This solution additionally includes an option where OAM provisioning is used to provide the H-ECS with the V-ECS information.

## 7.15	Solution #15: Initial EAS selection declaration

### 7.15.1	Architecture enhancements

None.

### 7.15.2	Solution description

This solution addresses KI#8 and KI#14. In this solution, the EES can know the selected EAS and the EES is enabled to trigger the EAS traffic influence after initial EAS is being determined.

Figure 7.15.2-1: Initial EAS selection declaration

In Figure 7.15.2-1 step 1 and 2, the EEC performs the start-up procedures for initial service provisioning and EAS discovery. EEC may send EAS discovery to multiple EESs. If registration is required by an EES, EEC registers into the EES before EAS discovery.

In step 3, the EEC (or AC and EEC) selects the initial EAS from the discovered EAS candidates.

In step 4, the EEC sends Selected EAS declaration request with AC ID, EAS ID, EAS endpoint and UE ID to the selected EES (which is determined based on the selected EAS).

The EES, in step 5:

-	may apply the EAS traffic influence with the N6 routing information of the EAS in the 3GPP Core Network, based on application KPIs and if the EAS traffic influence was not done before.

NOTE 1:	EES can also influence the EAS traffic in advance.

The EEC is then responded by the selected EES with success/failure of the request in step 6.

Editor's note:	It is FFS the timing for AC to connect to selected EAS.

NOTE 2:	The AC is not depicted in above figure for simplicity and solution to address interaction between AC and EEC is related to KI#4 for step 1 to 3.

Table 7.15.2-1 describes information elements for the selected EAS announcement request sent from the EEC to the serving EES.

Table 7.15.2-1: Selected EAS announcement request

| Information element | Status | Description |
| --- | --- | --- |
| UE ID | M | The identifier of the UE. |
| AC ID | O | The Application Client ID. |
| Security credentials | M | Security credentials. |
| Selected EAS ID | M | Selected EAS identifier. |
| Selected EAS Endpoint | M | Endpoint of the selected EAS. |

Currently, the EES can be aware of ongoing AC-EAS session if EAS uses EDGE-3 exposure APIs with UE IP address as input. The APIs are Eees_UEIdentifier and Eees_SessionWithQoS.

NOTE 3:	SSC creation requires EEC Context existence. The timing for SSC creation depends on when Eees_UEIdentifier or Eees_SessionWithQoS is invoked. It can also be possible that the SSC is not created at all during the entire AC-EAS communication period.

### 7.15.3	Solution evaluation

This solution addresses KI#14. It allows the EES to be aware of the selected EAS in initial service start so that the EES can control application traffic influence for the initial application traffic. This solution also addresses KI#8 about how the EES is aware of the application session information. It is a viable solution.

Editor's note:	Whether a new selected EAS declaration announcement API is needed or existing API (e.g. EEC registration) can be re-used is FFS.

## 7.16	Solution #16: EAS discovery for different users

### 7.16.1	Architecture enhancements

None.

### 7.16.2	Solution description

#### 7.16.2.1	General

The following solution corresponds to the key issue #12 on service differentiation for users.

In this solution, the User information corresponding to service permission level is introduced to achieve service differentiation.

The User information corresponding to service permission level could be a list of UE identifier(s) associated with service permission level. It enables EES support to handle the information of user priority or something like "allowed-list", or EAS deployment segmentation in EDN. And it is provided to EES by EAS during the EAS registration or EAS registration update.

Editor's note:	It is FFS where the User information can be stored and how to use the User information.

#### 7.16.2.2	Procedure

In this solution, when EES receives the EAS discovery request, in addition to service permission level filter, the EES identifies the EAS based on received User information.

Editor's note:	It is FFS if User information is not received in the EAS discovery request.

Editor's note:	It is FFS when large set of users are present, where the User information can be stored and how to use the User information.

### 7.16.3	Solution evaluation

This clause provides an evaluation of the solution.

## 7.17	Solution #17: Traffic influence for initial EAS discovery

### 7.17.1	Architecture enhancements

None.

### 7.17.2	Solution description

#### 7.17.2.1	General

The following solution corresponds to the key issue#14 on application traffic influence for initial EAS discovery clause 4.14.

#### 7.17.2.2	Procedure

In this solution, the EES will decide whether to perform traffic influence for the AC before the real data transmission between the AC and the EAS considering the AC type, requirement of the AC and so on, since AC may not connect to the EAS immediately after obtain the EAS information, in this case, the execution of traffic influence is unnecessary.

Figure 7.17.2.2-1: Traffic influence for initial EAS discovery

1.	The EEC determines to perform EAS discovery before the real data transmission between the AC and the EAS based on the response time in the AC profile. The EEC sends the EAS discovery request to the EES. The request contains the AC type, AC Service KPIs.

2.	Upon receiving the request from the EEC, if the AC has high requirement on response time, then the EES will select one EAS and determine to perform application traffic influence for this AC before the real data transmission between the AC and the EAS, based on the AC type, AC Service KPIs, response time. If the AC does not have high requirement on response time, then the EES will not perform application traffic influence immediately.

3.	The EES sends the AF request to the 5GC optimizing the user plane path between the UE and the EAS, if the EES determines to perform application traffic influence immediately.

4.	The EES sends the EAS discovery response to the EEC. The response contains the discovered EAS list with EAS being traffic influenced. The EEC sends the EAS discovery response to the AC.

### 7.17.3	Solution evaluation

The solution #17 is applicable to the scenario where the EES selects the EAS and performs the traffic influence immediately for the selected EAS after EEC sent EAS discovery request. It is a viable solution.

NOTE:	The EES may only be able to use the Service KPIs to determine whether to perform traffic influence immediately or not.

## 7.18	Solution #18: Constraint device in EDGEAPP

### 7.18.1	Architecture enhancements

None.

### 7.18.2	Solution description

This solution addresses KI#15. In this solution, in order to reduce power consumption in the UE, it specifies the EEC with Reduced Capabilities (RedEEC). The RedEEC skips EEC registration and the EAS discovery procedure is enhanced to delegate the EAS selection to EES. Figure 7.18.2-1 illustrates the details interactions.

Figure 7.18.2-1: service provisioning and EAS discovery for constraint device

In step 1, EEC performs service provisioning. For EAS discovery, the EEC sends the request to a selected EES from the candidate EES(s) and indicates UE type to EES in step 2. In step 3, the EES performs EAS discovery as described in clause 8.5.2.2 of 3GPP TS 23.558 [2] and in addition selects a suitable EAS from discovered candidate EAS(s) if UE type indicates constraint device, then the EES sends EAS discovery response to the EEC including the selected EAS information in the Discovered EAS list. In step 5, if the EAS discovery response contains successful result, the EEC sends to AC the received selected EAS information and the EAS discovery and selection procedure ends; otherwise, the EEC repeats step 2 with next selected candidate EES.

NOTE:	The EEC can interact with each candidate EES if no appropriate EAS can be selected by EES.

Table 7.18.2-1 and table 7.18.2-1 below show the detailed impact (highlighted with bold text) for EAS discovery request and response.

Table 7.18.2-1: EAS discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The ID of the requestor (e.g. EECID) |
| UE Identifier | O | The identifier of the UE (i.e. GPSI or identity token) |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| EAS discovery filters | O | Set of characteristics to determine required EASs, as detailed in Table 8.5.3.2-2. |
| UE location | O | The location information of the UE. The UE location is described in clause 7.3.2. |
| Target DNAI (NOTE) | O | Target DNAI information which can be associated with potential T-EAS(s) |
| EEC Service Continuity Support | O | Indicates if the EEC supports service continuity or not. The IE also indicates which ACR scenarios are supported by the EEC or, if this message is sent by the EEC to discover a T-EAS, which ACR scenario(s) are intended to be used for the ACR. |
| EES Service Continuity Support (NOTE) | O | The IE indicates if the S-EES supports service continuity or not. The IE also indicates which ACR scenarios are supported by the S-EES or, if the EAS discovery is used for an S-EES executed ACR according to clause 8.8.2.5, which ACR scenario is to be used for the ACR. |
| EAS Service Continuity Support (NOTE) | O | The IE indicates if the S-EAS supports service continuity or not. The IE also indicates which ACR scenarios are supported by the S-EAS or, if the EAS discovery is used for an S-EAS decided ACR according to clause 8.8.2.4, which ACR scenario is to be used for the ACR. |
| UE type | O | Indicates UE or device type (e.g. constraint device) |
| NOTE: This IE shall not be included when the request originates from the EEC. |  |  |

Table 7.18.2-2: EAS discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the EAS discovery request was successful. |
| > Discovered EAS list | O | List of discovered EAS(s). Each element includes the information described below. Based on UE type, only one EAS may be included. |
| >> EAS profile | M | Profile of the EAS. Each element is described in clause 8.2.4 |
| >> Lifetime | O | Time interval or duration during which the information elements in the EAS profile is valid and supposed to be cached in the EEC (e.g. time-to-live value for an EAS Endpoint) |
| Failure response | O | Indicates that the EAS discovery request failed. |
| > Cause | O | Indicates the cause of EAS discovery request failure. |

In service continuity, to offload UE from monitoring triggers (e.g. location change) to start ACR, EEC can announce its support for EDN side decided ACR scenarios (clause 8.8.2.4 and clause 8.8.2.5 of 3GPP TS 23.558 [2]) so that EEC only need to passively receive ACR information notifications.

### 7.18.3	Solution evaluation

This solution addresses KI#15 with EAS discovery and selection procedure for constrained device using EDGEAPP. The EEC relies on EES to select an appropriate EAS (the EEC does not need to select an EAS among discovered EAS list from candidate EESs).

## 7.19	Solution #19: EES determines the selected ACR scenario

### 7.19.1	Architecture enhancements

None.

### 7.19.2	Solution description

#### 7.19.2.1	General

The following solution corresponds to the key issue#19 on overlapping of ACR scenario in clause 4.19

#### 7.19.2.2	Procedure

In this solution, the EES is responsible to determine the selected ACR scenario for each AC based on the ACR scenarios supported by AC, EEC, EES and EAS. When the EES determines the selected ACR scenario for one AC, the EES will send the selected ACR scenario for the AC to the EEC and the EAS, then the EEC will notify the AC with the selected ACR scenario. With this solution the EES can determine which ACR approach can be enabled for the AC and EAS considering the detailed information (e.g EAS features, KPIs) of both the AC and the EAS and thus also avoiding the overlapping of ACR initiations by multiple entities which introduce unnecessary signalling to resolve co-existence issues.

Figure 7.19.2.2-1: EES determines the selected ACR scenario

1.	The EAS sends an ACR scenario selection subscription request to the EES.

2.	The EES sends an ACR scenario selection subscription response to the EAS with the subscription result.

3.	The EEC sends ACR scenario selection request to the EES, the request contains the AC service continuity support and EEC service continuity support indicating which ACR scenarios are supported by the AC and the EEC.

4.	Upon the EES obtains the AC service continuity support and EEC service continuity support from UE and EAS service continuity support, then the EES determines one suitable ACR scenario for the AC based on the ACR scenarios supported by AC, EEC, EES and EAS. The EES can select an appropriate ACR scenario for the AC from the intersection of the ACR scenarios supported by AC, EEC, EES and EAS.

NOTE 1:	The ACR scenario supported by the EAS is available in the EAS service continuity support of the EAS profile.

Editor's note: It is FFS how the EES can know the supported service continuity scenarios of the T-EES and T-EAS.

5.	The EES sends the ACR scenario selection notification to the EAS, the notification contains the selected ACR scenario and related ACID.

6.	The EES sends the ACR scenario selection response to the EEC including the selected ACR scenario and related ACID, then the EEC may notify the AC with the selected ACR scenario.

NOTE 2:	Using multiple ACR scenario can detect ACR timely.

### 7.19.3	Solution evaluation

It allows the EES to determine the selected ACR scenario. It is a viable solution. A new API (ACR scenario selection procedure) is used in this solution.

NOTE:	The selection of a single ACR scenario and therefore single ACR detection entity may not be suitable for time sensitive applications.

## 7.20	Solution #20: Propagation of EEL notifications to EEC using Edge Notification Server

### 7.20.1	Architecture enhancements

Architecture enhancement in clause 6.3 is the basis for this solution.

### 7.20.2	Solution description

#### 7.20.2.1	General

The following solution corresponds to the key issue #1, "enhanced notification service to the EEC" as described in clause 4.1.

In this solution, ENS is used as a centralized notification server enhancing EEL's notifications delivery mechanism to EEC. Such an architectural enhancement allows the EEC to inform EEL of its preferred method (e.g. Long-polling, WebSocket, FCM, APNS, OMA Push) of receiving notifications and enables Edge Application architecture to flexibly provide a diverse set of methods for notifications delivery to EEC.

#### 7.20.2.2	Notification delivery over a direct Notification Channel Procedure

In this procedure, based on EEC's request on the preferred method of notification delivery (i.e. Log-polling or WebSocket), a direct notification channel in between the EEC and ENS is established.

NOTE:	Notification delivery method using a Push server is described in clause 7.20.2.3.

Pre-conditions:

1.	EEC is aware of the ENS's endpoint through provisioning

2.	EES and ECS are authorized to interact with the ENS

3.	EEC is authorized to interact with the ENS, EES and ECS

Figure 7.20.2.2-1: Notification delivery over a direct Notification Channel

1.	The EEC sends a request to ENS for a direct notification channel (e.g. by indicating either a Long-polling (Pull) or a WebSocket (Push) notification delivery method).

2.	Upon receiving the request, the ENS performs an authorization check and further verifies if the requested notification delivery method (e.g. Long-polling or WebSocket ) can be used. If the request is authorized and the requested notification delivery method can be used, the ENS assigns a CallBack URL and an associated Channel URL for the EEC and stores the information for its subsequent use (i.e. any notification received at the assigned Callback URL will be made available over the associated Channel for consumption by the EEC).

3.	If the processing of the request was successful, the ENS responds with the assigned Callback URL and Channel URL

4.	EEC, upon receiving the Callback URL and Channel URL, based on the type of direct notification channel (e.g. Long-polling or WebSocket) it wishes to have with ENS, would either starts polling the Channel URL (e.g. HTTP GET Channel URL)f or events or uses the Channel URL and prepares it for the Push delivery (e.g. uses the channel URL and upgrades it to a WebSocket notification channel as per WebSocket protocol and in essence opens the notification channel for the flow of notifications as they arrive at ENS).

5.	The EEC sends a EAS discovery subscription request and/or ACR information subscription request (as described in clauses 8.5.3.4 and 8.8.4.8 of 3GPP TS 23.558 v17.0.0 respectively) to the EES with the Notification Target Address IE set to the CallBack URL received from the ENS in step 3 above.

6.	Upon receiving the request, the EES performs an authorization check (as described in clauses 8.5.2.3.2 or 8.8.3.5.2 of TS 23.558 depending on the subscription request in step 5) and if the request is authorized, the EES creates the associated subscription resource.

7	EES responds to EEC with the subscriptionId of the created resource.

8	The EEC sends a service provisioning subscribe request (as described in clause 8.3.3.3.4 of 3GPP TS 23.558 v17.0.0) to the ECS with the Notification Target Address IE set to the CallBack URL received from the ENS in step 3 above.

9	Upon receiving the request, the ECS performs an authorization check as described in clause 8.3.3.2.3.2 of TS 23.558 and if the request is authorized, the ECS creates the associated subscription resource.

10.	ECS responds to EEC with the subscriptionId of the created resource.

11.	Some events of interest to EEC occurs in ECS that satisfies trigger conditions for updating service provisioning of a subscribed EEC and the corresponding subscription directs the ECS to push the event to the CallBack URL which is terminated at the ENS.

12.	ECS pushes the notification to the CallBack URL

13.	ENS, upon receiving an event at the given CallBack URL, identifies the associated Channel URL (which it created and assigned to the EEC in step 2 above) and determines which notification delivery method was requested by the EEC (as per step 1 above). ENS forwards the notification to EEC either by responding to the Long-polling request (if EEC is using the Long-polling method) or pushes the notification to the EEC (if EEC is using a Push method such as the WebSocket delivery method),

14.	Some events of interest to EEC occurs in EES that satisfies trigger conditions for updating a subscribed EEC with the EAS discovery information or ACR related information (e.g. ACR complete event) and the corresponding subscription directs the EES to push the event to the CallBack URL which is terminated at the ENS.

15.	EES pushes the notification to the CallBack URL

16.	ENS, upon receiving an event at the given CallBack URL, identifies the associated Channel URL (which it created and assigned to the EEC in step 2 above) and determines which notification delivery method was requested by the EEC (as per step 1 above). ENS forwards the notification to EEC either by responding to the Long-polling request (if EEC is using the Long-polling method) or pushes the notification to the EEC (if EEC is using a Push method such as the WebSocket delivery method).

#### 7.20.2.3	Notification delivery using a Push Server Procedure (indirect Notification Channel)

In this procedure, based on EEC's request that the preferred method of notification delivery being via a Push server, there is no need for a direct notification channel between the EEC and ENS. Instead, ENS forwards the notifications through an identified Push server to the EEC. In other words, based on EEC's request, an indirect notification channel between the EEC and ENS with the Push server as an intermediary is established.

NOTE 1:	This procedure reuses the Push function setup steps 1a, 1b and 1c described in solution #1 (see clause 7.1.2.2).

Pre-conditions:

1.	EEC is aware of the ENS's endpoint through provisioning

2.	EES and ECS are authorized to interact with the ENS

3.	EEC is authorized to interact with the ENS, EES and ECS

4.	ENS is aware of the Push server endpoint and is authorized to interact with it

Figure 7.20.2.3-1: Notification delivery using a Push Server

1.	The EEC registers with the push function within the UE. The EEC acquires a push token and push server information from the push function. This step is depicted as sub-steps 1a, 1b and 1c in Figure 7.20.2.2-1.

NOTE 2:	The push server provides the push function in the UE with a push token, which is delivered to the EEC.

2.	The EEC sends a request to ENS for an indirect notification channel via the identified Push server. The request contains the Push server information as well as the EEC's push token (see step 1)

3.	Upon receiving the request, the ENS performs an authorization check and further verifies if the requested notification delivery method (i.e. Push server) can be used. If the request is authorized and the requested notification delivery method can be used, the ENS assigns a CallBack URL for the EEC and stores the information for its subsequent use (i.e. any notification received at the assigned Callback URL will be forwarded to the associated Push server for consumption by the EEC).

4.	If the processing of the request was successful, the ENS responds with the assigned Callback URL.

5.	The EEC sends a EAS discovery subscription request and/or ACR information subscription request (as described in clauses 8.5.3.4 and 8.8.4.8 of 3GPP TS 23.558 v17.0.0 respectively) to the EES with the Notification Target Address IE set to the CallBack URL received from the ENS in step 4 above.

6.	Upon receiving the request, the EES performs an authorization check (as described in clauses 8.5.2.3.2 or 8.8.3.5.2 of TS 23.558 depending on the subscription request in step 5) and if the request is authorized, the EES creates the associated subscription resource.

7.	EES responds to EEC with the subscriptionId of the created resource.

8.	The EEC sends a service provisioning subscribe request (as described in clause 8.3.3.3.4 of 3GPP TS 23.558 v17.0.0) to the ECS with the Notification Target Address IE set to the CallBack URL received from the ENS in step 4 above.

9.	Upon receiving the request, the ECS performs an authorization check as described in clause 8.3.3.2.3.2 of TS 23.558 and if the request is authorized, the ECS creates the associated subscription resource.

10.	ECS responds to EEC with the subscriptionId of the created resource.

11.	Some events of interest to EEC occurs in ECS that satisfies trigger conditions for updating service provisioning of a subscribed EEC and the corresponding subscription directs the ECS to push the event to the CallBack URL which is terminated at the ENS.

12.	ECS pushes the notification to the CallBack URL

13.	ENS, upon receiving an event at the given CallBack URL, identifies the associated Push server and the EEC's push token (which it received from EEC in step 2 above) and pushes the notification to the identified Push server using the given EEC's push token notification.

14.	Push server forwards the notification to the EEC via the push function.

15.	Some events of interest to EEC occurs in EES that satisfies trigger conditions for updating a subscribed EEC with the EAS discovery information or ACR related information (e.g. ACR complete event) and the corresponding subscription directs the EES to push the event to the CallBack URL which is terminated at the ENS.

16.	ECS pushes the notification to the CallBack URL

17.	ENS, upon receiving an event at the given CallBack URL, identifies the associated Push server and the EEC's push token (which it received from EEC in step 2 above) and pushes the notification to the identified Push server using the given EEC's push token notification.

18.	Push server forwards the notification to the EEC via the push function.

### 7.20.3	Solution evaluation

The proposed solution addresses Key Issue #1.

The Edge Notification Server enables an EDGEAPP architectural mechnism via which many different ways of pushing notifications to the EEC would become possible. Hence, the best notification delivery method  can be offered to the EEC/UE for the use case at hand as per EEC's request based on the given UE's limitations.

The Edge Notification Server should be viewed as an umbrella for different notification deliver methods to the EEC, one of which is solution #1 as per clause 7.1. Such an EDGEAPP architectural enhancement provides the needed flexibilty to handel different use cases while considering varrying UE's limitations.

The Edge Notification Server is considered an optional feature of the EDGEAPP architecture.

## 7.21	Solution #21: Prediction expiration time for service continuity planning enhancement

### 7.21.1	Architecture enhancements

None.

### 7.21.2	Solution description

#### 7.21.2.1	General

The following solution corresponds to the key issue #3 Enhancements to service continuity planning in clause 4.3.

In service continuity planning, the ACR is performed for a predicted or expected location of the UE in the future. Since the prediction is for a future location in a future time, a prediction expiration time is introduced to take account for the future time window in which the prediction is expected to happen. Therefore, in this solution, EEC includes its expectation or knowledge of the time that it expects the prediction to happen within the ACR request at latest. This is the time that the EEC requests the T-EAS to wait for the AC to connect at latest. The EESs can consider this time in deciding the time that they will wait for the ACR status update from the corresponding EASs.

EES can take into account the prediction expiration time in processing the ACR request message, e.g. accepting or rejecting the request. In case that the request is authorized and accepted by the EES, it also includes it in the notification (if any) to the EAS. Similarly, EAS can take into account the prediction expiration time in performing ACT. T-EAS and T-EES will receive the prediction expiration time and may consider it in deciding whether and for how long to wait for the UE to connect to them.

#### 7.21.2.2	Procedure

Figure 7.21.2.2-1 illustrates the EEC triggered ACR scenario for service continuity planning.

Pre-condition:

1.	The EEC has been authorized to communicate with the EES.

2.	EEC has detected the need for ACR for an expected/predicted location in the future.

Figure 7.21.2.2-1: EEC triggered ACR scenario for service continuity planning

1.	The EEC sends an ACR request message to the EES with initiation or determination action. Depending on the ACR scenario, the EES can be either of S-EES or T-EES.

In case of service continuity planning the ACR request message may include also the prediction expiration time to indicate the time that the EEC expects the prediction takes place at latest. This is the time that the EEC requests the T-EAS to wait for the AC to connect at latest.

2.	The EES checks if the requestor is authorized for this operation. If authorized, the EES processes the request and performs the required operations.

2a.	If authorized and if EES decides to continue the ACR, if the EAS notification indication is provided in the step 1 request and the EAS has subscribed to receive such notification, the EES shall notify the EAS about the need to start ACR. In that case, if the request in step 1 includes a prediction expiration time, the EES includes it in the notification to EAS.

If the request in step 1 is to S-EES this EAS is the S-EAS. Otherwise if the request of step 1 is to T-EES, this EAS is the T-EAS.

3.	The EES responds to the requestor's request, i.e. EEC, with an ACR response message.

4.	(a) If the action in the request in step 1 is initiation, the EES sends the information and parameters of the ACR to the remote EES. Otherwise if the action is determination the EES first discovers the T-EES and T-EAS, and then sends the information and parameters of the ACR to the T-EES.

(b) If the remote EAS has subscribed to receive such notification, the remote EES shall notify the EAS about the need to ACR and includes corresponding parameters of the ACR, e.g. prediction expiration time if it is provided in the request in step 1.

(c) The remote EES sends the response back to the EES to confirm it has received the ACR parameters.

If the request in step 1 is to S-EES the remote EES is the T-EES. Otherwise if the request of step 1 is to T-EES, the remote EES is the S-EES.

If the "Prediction expiration time" is not acceptable for the T-EAS, e.g. due to resource status, the EAS can reject the ACT. In that case the ACT fails.

5.	The ACR is executed depending on the ACR scenario. S-EAS and T-EAS perform application context transfer (ACT) in an application specific time and manner. The EAS may use the prediction expiration time received in step 2a to decide on whether and how to perform the ACT. The T-EAS may also consider it in deciding whether and for how long to wait for the AC of the UE to connect to it.

6.	The EAS sends the failure or success message in the ACR status update based on the results of step 5b. EAS can consider the prediction expiration time received in step 2a, if provided, to send the message in step 6.

If the request in step 1 is to S-EES this EAS is the S-EAS. Otherwise if the request of step 1 is to T-EES, this EAS is the T-EAS.

If the ACT as per step 5 is performed, and the AC (UE) does not connect to the T-EAS by "Prediction expiration time", the EAS can send ACT failure with the appropriate cause to the EES. In that case, the T-EAS can delete the transferred application context.

7.	The EES sends the ACR complete notify to the EEC with failure or success result. The EES also considers the failure or success message received in step 6 in the ACR complete notification.

Editor' note:	It is FFS how the EES can consider and process the "Prediction expiration time" IE in deciding details of ACR operations.

How the EES may react when the requested "Prediction expiration time" is not acceptable (e.g. based on ECSP policy, resource status) is FFS.

### 7.21.3	Solution evaluation

The Solution addresses KI#3 by including a prediction expiration time withing ACR request from the EEC. This information can be used by T-EAS to adjust its waiting time for the UE to reach the service area.

## 7.22	Solution #22: Support simultaneous EAS connectivity in ACR

### 7.22.1	Architecture enhancements

None.

### 7.22.2	Solution description

#### 7.22.2.1	Solution for traffic influence

To solve the issue about traffic influence to maintain both S-PSA and T-PSA to support simultaneous connectivity with both S-EAS and T-EAS during the service continuity, the AC includes the need for simultaneous EAS connectivity in the AC profile and the AC profile is sent to EEC via EDGE-5 reference point so that the EEC can request such need in the ACR request sent to EES. Then the EES, at the time of requesting traffic influence towards 3GPP CN, provides additional requirement for simultaneous PSA connectivity as described in clause 6.3.4 of 3GPP TS 23.548 [19].

Figure 7.22.2.1-1 describes the detailed interaction between the AC and EEC for AC registration procedure (a possible way to provide AC profile to EEC).

Figure 7.22.2.1-1: AC Registration procedure

1.	The AC sends AC registration request to EEC, the request includes the AC profile (see table 7.22.2.1-1).

2.	The EEC authorizes the AC registration request and stores the AC profile.

3.	The AC is responded with AC registration response (success/failure).

Table 7.22.2-1: AC Profile

| Information element | Status | Description |
| --- | --- | --- |
| ACID | M | Identity of the AC. |
| AC Type | O | The category or type of AC (e.g. V2X). This is an implementation specific value. |
| Preferred ECSP list | O | When used in a service provisioning request, this IE indicates to the ECS which ECSPs are preferred for the AC. The ECS may use this information in the selection of EESs. |
| AC Schedule | O | The expected operation schedule of the AC (e.g. time windows) |
| Expected AC Geographical Service Area | O | The expected location(s) (e.g. route) of the hosting UE during the AC's operation schedule. This geographic information can express a geographic point, polygon, route, signalling map, or waypoint set. |
| AC Service Continuity Support | O | Indicates if service continuity support is required or not for the application. The IE also indicates which ACR scenarios are supported by the AC and which of these are preferred by the AC. |
| Simultaneous EAS connectivity information in service continuity | O | Indicates if simultaneous EAS connectivity is needed and the inactive time guidance for keeping connectivity towards the S-EAS. |
| List of EASs | O | List of EAS that serve the AC along with the service KPIs required by the AC |
| > EASID | M | Identifier of the EAS |
| > Expected AC Service KPIs | O | KPIs expected in order for ACs to receive currently required services from the EAS |
| > Minimum required AC Service KPIs | O | Minimum KPIs required in order for ACs to receive meaningful services from the EAS |

Figure 7.22.2.1-2 describes the detailed interaction between the EEC and EES during service continuity (i.e. ACR).

Figure 7.22.2.1-2: ACR initiation procedure

0.	The EEC is informed about AC profiles (e.g. during AC registration procedure as depicted in Figure 7.22.2.1-1).

1.	The EEC triggers ACR request (type: initiation) to EES, the request includes the simultaneous EAS connectivity information in service continuity (see table 7.22.2.1-2) which was previously received as part of the AC profile (see table 7.22.2.1-1).

2.	The EES authorizes the ACR request. Then the EES may use information provided in the request to apply the AF traffic influence with the N6 routing information of the T-EAS and Simultaneous EAS connectivity information in the 3GPP Core Network (if applicable), as described in clause 5.6.7.1 of 3GPP TS 23.501 [5] and clause 6.3.4 of 3GPP TS 23.548 [19].

Editor's note:	Since the 3GPP CN only supports simultaneous PSA connectivity in SSC mode 3 or session breakout, it is FFS whether EES should firstly know PDU session capability before invoking AF traffic influence API.

3.	The EEC is responded with ACR response (success/failure).

Table 7.22.2.1-2: ACR request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor Identifier | M | Unique identifier of the requestor (i.e. EECID or EASID). |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| EASID | O | Identifier of the EAS |
| UE identifier | O | The identifier of the UE (i.e. GPSI). |
| ACID | O | The identifier of the AC. |
| ACR action | M | Indicates the ACR action (ACR initiation or ACR determination) |
| ACR initiation data | O | ACR initiation IEs to be included in an ACR request message when ACR action indicates it is ACR initiation request. |
| > T-EAS Endpoint | M | Endpoint information (e.g. URI, FQDN, IP 3-tuple) of the T-EAS. |
| > Previous T-EAS Endpoint (NOTE 7) | O | Endpoint information (e.g. URI, FQDN, IP 3-tuple) of the T-EAS of the previous ACR. |
| > DNAI of the T-EAS | O | DNAI information associated with the T-EAS. |
| > N6 Traffic Routing requirements | O | The N6 traffic routing information and/or routing profile ID corresponding to the T-EAS DNAI. |
| > Simultaneous EAS connectivity information | O | Indicates if simultaneous EAS connectivity is needed and the inactive time guidance for keeping connectivity towards the S-EAS. |
| > EAS notification indication | M | Indicates whether to notify the EAS about the need of ACR. |
| … | … | … |
| > S-EAS endpoint | O | Endpoint information of the S-EAS |
| ACR determination data | O | ACR determination IEs to be included in an ACR request message when ACR action indicates it is ACR determination request. |
| > S-EAS endpoint | M | Endpoint information of the S-EAS |

### 7.22.3	Solution evaluation

This solution addresses KI#4 about "Whether and how an AC registers to an EEC". During the AC registration, the AC is able to indicate its desire for keeping Simultaneous EAS Connectivity. This solution addresses KI#21 about traffic influence. The EES interacts with 3GPP CN to satisfy AC need during ACR request processing.

## 7.23	Solution #23: UE identification with NAT

### 7.23.1	Architecture enhancements

None.

### 7.23.2	Solution description

#### 7.23.2.1	General

This solution corresponds to KI#16 on support of NAT deployed within the edge data network.

In this solution, the EEC provides either the CN network assigned IP address (i.e., the private IP address of the UE) or its UE ID (if it already has one) to the EES to obtain the UE identifier (Edge UE ID) from the EES. If the UE ID is not provided, EES uses the private IP address of the UE provided by the EEC to invoke the CN capability to translate UE's IP address to its UE ID (e.g., UE'sExternal UE ID).

Once obtained, the EEC passes the Edge UE ID to the AC, which provides it to the EAS for use over EDGE-3 interface. This also allows the UE to protect its privacy by providing an option to obtain from the EES an Edge UE ID to share with the EAS(s) via the AC(s) instead of exposing its UE ID (if it already has one) to the AC(s). With EAS and EES both being aware of the Edge UE ID, issues pertaining to NAT and UE identification with privacy are resolved.

NOTE 1:	The UE ID provided by EEC is in the form of MSISDN.

NOTE 2:	SA3 coordination may be needed corresponding to privacy.

#### 7.23.2.2	Procedure

Figure 7.23.2.2 illustrates the procedure for the EEC to obtain the Edge UE ID from the EES and provide to the AC.

Figure 7.23.2.2-1: EEC obtaining UE ID from the EES

1.	AC sends an Edge UE ID request to the EEC. The request may include the list of EASIDs for which the AC is requesting the Edge UE ID information.

2.	The EEC upon receiving the request validates if AC is authorized to request this information. If AC is authorised, the EEC sends the Edge UE ID request to the EES. The request includes either the CN assigned private IP address of the UE or its UE ID (if it already has one) and may include the list of EASIDs if provided by the AC.

NOTE 1:	EEC can also send this request without receiving a request from AC in step 1.

NOTE 2:	Private IP address used by AC and private IP address used by EEC can be the same or different (if different PDU sessions are used). The request from EEC can include either of the IP addresses.

3.	Upon receiving the request from EEC, the EES authorizes the EEC. If authorized and the UE ID is not included in the request the EES invokes the CN capability Nnef_UEId_Get for translating the UE's IP address to its UE ID as defined in 3GPP TS 23.502 [08] clause 4.15.10. If the request from EEC includes a list of EASIDs, the EES may invoke the Nnef_UEId_Get API for each EAS individually to obtain EAS specific UE ID(s). The EES generates temporary Edge UE ID(s) which may be the same as the 3GPP CN provided UE ID or may be assigned by the EES itself. If UE ID is included in the request received from EEC, the EES generates temporary Edge UE ID. The temporary Edge UE ID may be specific for the EASs included in request received from EEC in step 2, in which case upon receiving a request on EDGE-3 interface, the EES matches the EASID in the request with EASIDs to which the Edge UE ID was assigned before processing the request.

4.	The EES sends the Edge UE ID response to the EEC including the Edge UE ID(s).

5.	Upon receiving the response from the EES, the EEC provides the Edge UE ID information to the AC by sending the Edge UE ID response.

6.	The AC provides the Edge UE ID information to the EAS.

NOTE 3:	Details on how the AC provides the information to the EAS is out of scope.

7.	Once received from the AC, the EAS uses the Edge UE ID to invoke the APIs provided by the EES over EDGE-3 interface (e.g., T-EAS Discovery, UE location request, ACR request, and EELManangedACR services).

8.	Once received from the EES, the EEC uses the Edge UE ID to invoke API provided by the EES over EDGE-1 interface (e.g., EAS Discovery and ACR request services).

9.	The EES uses the UE ID received from the EEC or obtained from the CN in step 3 to invoke the 3GPP CN capabilities as in TS 23.558, Clause 8.10.3. This step can be performed following triggers that require 3GPP CN capabilities to be invoked (e.g., on receiving a request over EDGE-1 or EDGE-3), in which case, to invoke the 3GPP CN capabilities the EES uses the UE ID associated with the Edge UE ID included in the trigger.

NOTE 4:	This procedure has impacts on EDGE-5 interface.

### 7.23.3	Solution evaluation

This solution solves open issues of KI#16. In this solution EES translates the EEC provided UE's private IP address to its UE ID using the CN provided capability (Nnef_UEId_Get). This UE ID is then shared back to the EEC, which provides it to the AC. AC can then share the UE ID with the EAS for use over EDGE-3 interface.

The solution also allows the EES to convert the CN or EEC provided UE ID to Edge UE ID, which is managed by the EES. Edge UE ID ensures privacy of the UE ID and allows the EES to assign EAS specific IDs for controlled access over EDGE-3 interface.

EEC can request the Edge UE ID on its own, this allows the EEC to use the received Edge UE ID over EDGE-1 interface.

In some network deployments, there may be a private IP address overlap issue where more than one UE is assigned the same private IP address. Support from SA2 is required in such cases.

Coordination with SA3 is required to check whether there is a security issue if the EEC shares its private IP address or MSISDN with a trusted 3rd party EES.

In this solution, whether EESs use the same or different algorithms to generate Edge UE ID and weather the Edge UE ID should be unique should be specified in normative work.

## 7.24	Solution #24: ACR between EAS and CAS with CES

### 7.24.1	Architecture enhancements

Architecture enhancements in clause 6.6 is the basis for this solution.

### 7.24.2	Solution description

#### 7.24.2.0	General

Figure 7.24.2-2: UE moves from one EDN to Cloud DN then to another EDN

As depicted in figure 7.24.2-2, since the EAS may have service area restriction, once the UE is moving out of the current edge coverage, to keep service continuity, the application client needs to connect to either another EAS in new EDN or the CAS. For the latter case, the EES interacts with the CES via EDGE-9' reference point and application context is transferred between the EAS and CAS. Later, if the UE is moving to an area with edge coverage, the CES interacts with the EES via EDGE-9' reference point and application context is transferred between the CAS and EAS.

#### 7.24.2.1	ACR Scenarios

The ACR scenarios in TS 23.558 can be extended to include ACR between EAS and CAS. The extension would also include extensions to relevant procedures used in the ACR Scenarios (e.g.  T-EAS discovery, ACR request).

##### 7.24.2.1.1	CAS decided ACR scenario

TS 23.558 clause 8.8.2.4 "S-EAS decided ACR scenario" can be updated to allow "CAS decided ACR scenario". The S-EAS can be the CAS, the S-EES can be the CES when ACR happens between CAS and EAS. The CES will, therefore, need to be part of the EEL that facilitate the discovery of the T-EAS by interacting with the ECS.

Since in this procedure T-EAS discovery is used by the CAS – in step 3 – to discover the T-EAS the procedure in TS 23.558 clause 8.8.3.2 "Discover T-EAS" is also updated as described in clause 7.25.2.2.2.

##### 7.25.2.2.2	 "Discover T-EAS" for CAS

TS 23.558 clause 8.8.3.2 "Discover T-EAS" can be updated to allow CAS to discover T-EAS. The S-EAS can be the CAS, the S-EES can be the CES when ACR happens between CAS and EAS. Therefore, the CES has to be part of the EEL to facilitate the discovery of the T-EAS by interacting with the ECS.

### 7.24.3	Solution evaluation

This solution addresses KI#11. The introduction of CES deployed in the central DN enables application service continuity using EDGEAPP mechanisms.

The solution provides ACR scenario parity support for relocating application session between EAS and CAS, existing 5 ACR scenarios (including service continuity planning) can be supported with introduction of CES.

## 7.25	Solution #25: ACR between EAS and CAS without CES

### 7.25.1	Architecture enhancements

Architecture enhancements in clause 6.5 is the basis for this solution.

### 7.25.2	Solution description

#### 7.25.2.1	General

The following solution addresses open issues of key issue #11, ACR between EAS and Cloud Application Server.

#### 7.25.2.2	Procedure

The scenarios specified in 3GPP TS 23.558 (Rel-17) clause 8.8 have been updated to consider the ACR between EAS and Cloud Application Server.

##### 7.25.2.2.1	Updated 3GPP TS 23.558 clause 8.8.2.2 Initiation by EEC using regular EAS Discovery

The scenario handles ACR as a result of the UE moving to, or the UE expecting to move to, a new location which is outside the service area of the serving EAS. It further relies on the EEC being triggered as a result of the UE's movement.

This scenario is based on Service Provisioning (as specified in TS 23.558) and DNS procedures to discover the CAS that shall serve the AC as a result of the UE's new location, and that shall receive the Application Context from the serving EASs. The scenario below describes the relocation of a single application context to a CAS. However, it should be repeated for each active AC in the UE for which EAS or EDN is not available on that UE location.

Figure 7.25.2.2.1-1: Updated 3GPP TS 23.558 Figure 8.8.2.2-1: ACR initiated by the EEC and AC

The pre-conditions:

1.	The AC in the UE already has a connection to a corresponding S-EAS;

2.	The preconditions for the Service provisioning - Request/Response model as specified in TS 23.558 with regards to the EEC are fulfilled; and

3.	The EEC is triggered when it obtains the UE's new location or is triggered by another entity such as an ECS notification.

Phase I: ACR Detection

1.	The EEC detects the UE location update as a result of a UE mobility event and is provided with the UE's new location as described in TS 23.558. The EEC can also detect an expected or predicted UE location in the future as described in TS 23.558.

NOTE 1:	If the EEC is triggered by an external entity such as by a notification from the ECS, unavailability of new EESs (to be used as T-EESs) is provided by that notification and step 3 below is skipped.

Phase II: ACR Decision

2.	Either the AC or the EEC makes the decision to perform the ACR.

NOTE 2:	Which applications require ACR can be decided based on the application profile, e.g. requirement of service continuity of the application.

Phase III: ACR Execution

3.	The EEC performs Service Provisioning (as specified in TS 23.558) for all active applications that require ACR. Since the location of the UE has changed, this procedure results in unavailability of T-EESs that are relevant to the supplied applications and the new location of the UE. If the service provisioning is done without supplying the application information but EES information is provisioned, the EEC attempts discovering relevant T-EAS with the EES provisioned in the service provisioning response, if any. Service provisioning or discovery of relevant T-EAS may not result in EES configuration or T-EAS is not discovered respectively.

If the change in UE's location does not trigger a need to change the serving EAS, the subsequent steps will not take place. The EEC remains connected to the serving EESs and the ACs remain connected to their corresponding serving EASs.

4.	If the change in the UE's location triggers a need to change the S-EAS but the EEC is not provided with a T-EAS, the EEC informs the AC of the unavailability of a suitable EDN for the new location of the UE.

5.	The AC triggers the UE to perform DNS resolution for the CAS relevant for the AC. The UE may need to establish a new PDU connection to the CAS.

6a.	The AC sends ACR request to the EEC and the EEC responds ACR response to the AC.

6b.	The EEC performs ACR launching procedure (as described in TS 23.558) to the S-EES with the ACR action indicating ACR initiation and the corresponding ACR initiation data (along with the details of the CAS and without the need to notify the EAS). The S-EES may apply the AF traffic influence with the N6 routing information of the CAS in the 3GPP Core Network (if applicable), as described in TS 23.558.

7.	The AC is triggered by the EEC to start ACT. The AC decides to initiate the transfer of application context from the S-EAS to the CAS.

After the ACT is completed, the AC remains connected to the CAS and disconnects from the S-EAS; the EEC is informed of the completion of the ACT.

The S-EAS or CAS can further decide to terminate the ACR, and the CAS can discard the application context (e.g. based on monitoring the location of the UE). It is up to the implementation of the S-EAS and CAS whether and how to make such a decision.

Phase IV: Post-ACR Clean up

8.	The S-EAS sends the ACR status update message to the S-EES as specified in TS 23.558.

9.	If the status in step 8 indicates a successful ACT, the S-EES sends the ACR information notification (ACR complete) message to the EEC to confirm that the ACR has completed as specified in TS 23.558.

The CAS can perform the required CN capability exposure subscriptions upon receiving the application context.

##### 7.25.2.2.2	Updated 3GPP TS 23.558 clause 8.8.2.3 EEC executed ACR via S-EES

In this scenario, the EEC is triggered as a result of the UE's movement as described in 8.8.1.1 of TS 23.558. Figure 7.25.2.2.2-1 illustrates the EEC executing ACR via the S-EES.

Pre-condition:

1.	The AC at the UE already has a connection to the S-EAS; and

2.	The EEC is able to communicate with the S-EES.

Figure 7.25.2.2.2-1: Updated 3GPP TS 23.558 Figure 8.8.2.3-1: EEC executed ACR

Phase I: ACR Detection

1.	The EEC detects that ACR may be required as described in clause 8.8.1.1 of TS 23.558. The EEC may detect that ACR may be required for an expected or predicted UE location in the future as described in clause 8.8.1.1 of TS 23.558.

Phase II: ACR Decision

2.	The EEC decides to proceed required procedures for triggering ACR.

Phase III:	ACR Execution

3.	The EEC performs Service Provisioning (as specified in TS 23.558) for all active applications that require ACR. Since the location of the UE has changed, this procedure results in unavailability of T-EESs that are relevant to the supplied applications and the new location of the UE, as per the assumption of this scenario. Service provisioning or discovery of relevant T-EAS may not result in EES configuration or T-EAS is not discovered respectively. The AC triggers the UE to perform DNS resolution for the cloud application server relevant for the AC. The UE may need to establish a new PDU connection to the CAS.

NOTE 1:	Several EEC registrations with different EESs may result from T-EAS discovery process during a single ACR operation.

Editor's note:	If the service provisioning is done without supplying the application information, the EEC attempts discovering relevant T-EAS with the EES provisioned in the service provisioning response is FFS

4.	The EEC performs ACR launching procedure (as described in clause 8.8.3.4 of TS 23.558) to the S-EES with the ACR action indicating ACR initiation and the corresponding ACR initiation data (along with the details of the CAS and with the need to notify the EAS). The S-EES authorises the request from the EEC. The S-EES decides to execute ACR based on the information received from the EEC and/or EAS profile. The S-EES may apply the AF traffic influence with the N6 routing information of the CAS in the 3GPP Core Network (if applicable) and sends the ACR management notification for the "ACT start" event to the S-EAS, as described in clause 8.6.3, to initiate ACT between the S-EAS and the CAS. If the EEC has not subscribed to receive ACR information notifications for ACR complete events from the S-EES, the EEC subscribes for the notifications as described in clause 8.8.3.5.2 of TS 23.558.

5.	The S-EAS transfers the application context to the CAS at implementation specific time. This process is out of scope of the present specification.

NOTE 2:	The S-EAS or CAS can further decide to terminate the ACR, and the CAS can discard the application context based on information received from EEL and/or other methods (e.g. monitoring the location of the UE). It is up to the implementation of the S-EAS and CAS whether and how to make such a decision.

Phase IV:	Post-ACR Clean up

6. The S-EAS sends the ACR status update message to the S-EES as specified in clause 8.8.3.8 of TS 23.558.

7. If the status in step 7 indicates a successful ACT, the S-EES sends the ACR information notification (ACR complete) message to the EEC to confirm that the ACR has completed as specified in clause 8.8.3.5.3 of TS 23.558.

Editor's note:	CAS performing the required CN capability exposure subscriptions upon receiving the application context is FFS.

##### 7.25.2.2.3	Updated 3GPP TS 23.558 clause 8.8.2.4 S-EAS decided ACR scenario

In this scenario, the S-EAS may detect the need of ACR locally or is notified by the S-EES via ACR management notifications for "ACR monitoring" events. The S-EAS make the decision about whether to perform the ACR, and starts the ACR at a proper time.

Pre-conditions:

1.	The S-EAS may depend on the receipt of certain User plane path management events from the S-EES, e.g. "user plane path change" events or "ACR monitoring" events, to detect the need for an ACR. For the following procedure it is assumed that the S-EAS has subscribed to continuously receive the respective events from the S-EES; and

2.	The EEC has subscribed to receive ACR information notifications for target information notification events and ACR complete events from the S-EES, as described in clause 8.8.3.5.2 of TS 23.558.

Figure 7.25.2.2.3-1: Updated 3GPP TS 23.558 Figure 8.8.2.4-1: S-EAS decided ACR

S-EAS decided ACR is outlined with four main phases: detection, decision, execution and clean up.

Phase I: ACR Detection

1.	The S-EAS either receives ACR management notifications from source Edge Enabler Sever indicating that ACR may be required ("ACR monitoring" event), or self detects the need for ACR (e.g. upon receipt of a "user plane path change" event). If the ACR management notification indicates "ACR monitoring" event, then the notification will also contain the CAS information (see clause 8.6.3.2.3 of TS 23.558). The S-EAS may detect that ACR may be required for an expected or predicted UE location in the future as described in clause 8.8.1.1 of TS 23.558.

NOTE 1:	How the S-EAS self detects the local need for ACR is outside the scope of this specification.

Phase II: ACR Decision

2.	The S-EAS makes the decision to perform the ACR

NOTE 2:	How the S-EAS determines when to start the ACR is outside the scope of this specification.

Phase III:	ACR Execution

3.	If the ACR required is self detected, the S-EAS requests the S-EES to discover the targets as described in TS 23.558. When S-EES determines that no relevant EAS is available for the UE's location it finds out the details of the CAS, e.g. via DNS query/discovery, and provides the details of the CAS to the S-EAS. After S-EAS determines to use CAS, the S-EAS may apply the AF traffic influence with the N6 routing information of the CAS in the 3GPP Core Network (if applicable).

NOTE 3:	EAS endpoint in discovery could be a FQDN of CAS, identical with the FQDN used in DNS query.

4.	The S-EAS sends selected CAS declaration message to S-EES, to inform S-EES the determined CAS to use as described in clause 8.8.3.7 of TS 23.558.

5.	Based on the CAS selection information received from the S-EAS, the S-EES sends the target information notification to the EEC as described in clause 8.8.3.5.3 of TS 23.558.

6.	The S-EAS transfers the application context to the CAS selected in step 3. This process is out of scope of the present specification.

NOTE 3:	The S-EAS or CAS can further decide to terminate the ACR, and the CAS can discard the application context based on information received from EEL and/or other methods (e.g. monitoring the location of the UE). It is up to the implementation of the S-EAS and CAS whether and how to make such a decision.

Phase IV:	Post-ACR clean up

7.	The S-EAS sends the ACR status update message to the S-EES as specified in clause 8.8.3.8 of TS 23.558.

8. If the status in step 8 indicates a successful ACT, the S-EES sends the ACR information notification (ACR complete) message to the EEC to confirm that the ACR has completed as specified in clause 8.8.3.5.3 of TS 23.558.

Editor's note:	CAS performing the required CN capability exposure subscriptions upon receiving the application context is FFS.

##### 7.25.2.2.4	Updated 3GPP TS 23.558 clause 8.8.2.5 S-EES executed ACR

Figure 7.25.2.2.4-1 illustrates the S-EES detecting, deciding and executing ACR from the S-EAS to the CAS. This may include EELManagedACR by S-EES when initiated by S-EAS as per clause 8.8.3.6 of TS 23.558.

Pre-condition:

1.	The AC at the UE already has a connection to the S-EAS;

2.	The EEC is able to communicate with the S-EES;

3.	The EEC has subscribed to receive ACR information notifications for target information notification events and ACR complete events from the S-EES, as described in clause 8.8.3.5.2 of TS 23.558;

4.	The S-EAS optionally subscribed to receive ACR management notifications for "ACR facilitation" events to the S-EES, in order to enable detection at S-EAS.

5.	In case of EELManagedACR, the CAS has subscribed to receive ACT status notifications as described in clause 8.8.3.6.2.3 of TS 23.558.

Figure 7.25.2.2.4-1: Updated 3GPP TS 23.558 Figure 8.8.2.5-1: S-EES executed ACR

1.	The S-EAS may initiate EELManagedACR with S-EES as specified in clause 8.8.3.6 of TS 23.558. In this step, the S-EAS and S-EES negotiate an address of the Application Context storage to S-EES. The S-EAS puts the Application Context at this address which can be further accessed by the S-EES when the ACT is required.

In this case, the S-EES executes steps 2 (i.e., S-EES detection), 4, 5, 6, 7, 8, 9 and 11. Rest of steps are skipped.

Phase I: ACR Detection

2.	Detection entities (S-EAS, S-EES, EEC) detect that ACR may be required as described in clause 8.8.1.1 of TS 23.558. The detection by the S-EES may be triggered by the User Plane path change notification received from the 3GPP Core Network due to S-EAS request for "ACR facilitation" event (see clause 8.6.3 of TS 23.558) or due to step 1.

The detection entity may detect that ACR may be required for an expected or predicted UE location in the future as described in clause 8.8.1.1 of TS 23.558.

Phase II: ACR Decision

3.	The detection entity performs ACR launching procedure (as described in clause 8.8.3.4 of TS 23.558) with the ACR action indicating ACR determination and the corresponding ACR determination data.

4.	The S-EES authorises the message if received. The S-EES decides to execute ACR based on the information received or local detection, and the information of EEC context or EAS profile, and then proceed the below steps.

Phase III:	ACR Execution

5.	The S-EES determines the targets via the Discover T-EAS procedure in clause 8.8.3.2 of TS 23.558. When S-EES determines that no relevant EAS is available for the UE's location it finds out the details of the CAS, e.g. via DNS query/discovery.

6.	The S-EES sends the target information notification to the EEC as described in clause 8.8.3.5.3 of TS 23.558.

7.	The S-EES may apply the AF traffic influence with the N6 routing information of the CAS in the 3GPP Core Network (if applicable).

8.	The S-EES sends the ACR management notification (e.g. as notification for "ACR facilitation" event or "ACT start" event as described in clause 8.6.3 or due to step 1) to the S-EAS to initiate ACT between the S-EAS and the CAS.

9.	The Application Context is transferred from S-EAS to the CAS at implementation specific time. In the case of EELManagedACR, the S-EES accesses the Application Context from the address as per step 1 and the S-EES either engage in the ACT from S-EAS to the CAS (obtained as per step 5) in a secure way or S-EES shares the storage location of the Application Context with the CAS. Further the CAS accesses the Application Context. The S-EAS may also perform the ACT directly with CAS, the specification of such process is out of scope of the present document.

NOTE 1:	The Application Context is encrypted and protected by the application layer. The S-EES engages in the packet level transport of the Application Context and has no visibility to the content of the Application Context.

NOTE 2:	The S-EAS or CAS can further decide to terminate the ACR, and the CAS can discard the application context based on information received from EEL and/or other methods (e.g. monitoring the location of the UE). It is up to the implementation of the S-EAS and CAS whether and how to make such a decision.

Phase IV:	Post-ACR Clean up

10.	The S-EAS sends the ACT status update message to the S-EES as specified in clause 8.8.3.8 of TS 23.558.

11.	If the status in step 10 indicates a successful ACT, the S-EES sends the ACR information notification (ACR complete) message to the EEC to confirm that the ACR has completed as specified in clause 8.8.3.5.3 of TS 23.558.

NOTE 3:	The Application Client mechanism to support switchover of the application traffic to CAS is out of scope of the specification.

##### 7.25.2.2.5	EEC initiated ACR

In this case, when AC is currently served by a CAS, the EEC detects the need for ACR and makes the decision about whether to perform the ACR and starts the ACR at a proper time.

The EEC detects the need for ACR and decides to trigger ACR. If the EEC has a valid S-EES information (i.e. AC connected to an EAS before connecting to the CAS), the procedure is similar to the "Initiation by EEC using regular EAS Discovery" and "EEC executed ACR scenario via S-EES" as specified in TS 23.558 [2], clause 8.8.2.2 and clause 8.8.2.3; otherwise, the procedure is similar as "EEC executed ACR scenario via T-EES" as specified in TS 23.558 [2] clause 8.8.2.6. The procedures are with the difference that the CAS replaces the S-EAS.

##### 7.25.2.2.6	CAS initiated ACR

In this scenario, the CAS detects the need for ACR and makes the decision about whether to perform the ACR and starts the ACR at a proper time.

When ACR happens between EAS and CAS, the S-EAS can be the CAS. During the ACR execution phase, the CAS needs to know the EES before continuing with T-EAS discovery. Once the CAS knows the EES, the T-EAS discovery and the remaining steps are similar to the "S-EAS decided ACR scenario" as specified in TS 23.558 clause 8.8.2.4, where the CAS acts like the S-EAS.

Editor's Note: How CAS detects ACR is FFS

Editor's Note: How CAS knows the EES is FFS.

Editor's Note: Whether the UE Identifier need to be shared with the CAS and how it is shared is FFS.

### 7.25.3	Solution evaluation

This clause provides an evaluation of the solution.

## 7.26	Solution #26: Bundled EASs

### 7.26.1	Architecture enhancements

None.

### 7.26.2	Solution description

#### 7.26.2.1	General

This solution corresponds to KI#18 on linkages between EASs.

Editor's Note:	The solution assumes that the EASs belonging to a bundle is part of the same EDN. Support for EASs spread across EDNs is FFS.

The solution extends the AC profile and EAS profile to introduce the following new IEs and their handling by different EEL functions:

-	EAS bundle information: EAS bundle information (e.g., EAS bundle ID, information of associated EASs) establishes an association between the EASs. Edge Enabler Layer handles the Edge Application Servers belonging to the same bundle as required by the bundle requirements. When included in the EAS profile, EAS Bundle information denotes the bundle to which the EAS belongs. When included in the AC profile EAS Bundle information is used to perform different Edge Enabler Layer operations, such as EAS discovery.

-	EAS bundle requirements: This IE provides the Edge Enabler Layer the requirements that apply to the bundle of EAS. The requirements may include combined discovery and combined ACR.

Both, EAS bundle information and EAS bundle requirements, are provided by the ASP. EAS bundle information can be a list of EASs or a bundle ID, however, detailed format of the EAS bundle ID is out of scope.

Table 7.26.2.1-1: AC Profile

| Information element | Status | Description |
| --- | --- | --- |
| ACID | M | Identity of the AC. |
| AC Type | O | The category or type of AC (e.g. V2X). This is an implementation specific value. |
| Preferred ECSP list | O | When used in a service provisioning request, this IE indicates to the ECS which ECSPs are preferred for the AC. The ECS may use this information in the selection of EESs. |
| AC Schedule | O | The expected operation schedule of the AC (e.g. time windows) |
| Expected AC Geographical Service Area | O | The expected location(s) (e.g. route) of the hosting UE during the AC's operation schedule. This geographic information can express a geographic point, polygon, route, signalling map, or waypoint set. |
| AC Service Continuity Support | O | Indicates if service continuity support is required or not for the application. The IE also indicates which ACR scenarios are supported by the AC and which of these are preferred by the AC. |
| List of EASs | O | List of EAS that serve the AC along with the service KPIs required by the AC |
| > EASID | M | Identifier of the EAS |
| > Expected AC Service KPIs | O | KPIs expected in order for ACs to receive currently required services from the EAS, as described in Table 8.2.3-1 |
| > Minimum required AC Service KPIs | O | Minimum KPIs required in order for ACs to receive meaningful services from the EAS, as described in Table 8.2.3-1 |
| EAS bundle information | O | EAS bundle which AC requires. |
| EAS bundle requirements | O | Requirements associated with the EAS bundle. |
| > Coordinated EAS discovery | O | Indicates if AC requires coordinated EAS discovery. |
| > Coordinated ACR | O | Indicates if AC requires coordinated ACR. The IE may further indicate what actions must be taken if ACR for one or more bundled EAS fails e.g., ACR for all other EAS that are part of the bundle must be cancelled or not. |

Table 7.26.2.1-2: EAS Profile

| Information element | Status | Description |
| --- | --- | --- |
| EASID | M | The identifier of the EAS |
| EAS Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EAS. This information maybe discovered by EEC and exposed to ACs so that ACs can establish contact with the EAS. |
| List of EAS bundle information | O | List of EAS bundles to which the EAS belongs and related bundling requirements. |
| > EAS bundle information | M | Identifier of the bundle to which EAS belongs. |
| > EAS bundle requirements | O | Requirements associated with the bundle |
| >> Coordinated EAS discovery | O | Indicates if EAS discovery request for one of the bundled EAS is received, then EAS discovery response should include information of all the EASs belonging to the bundle. |
| >> Coordinated ACR | O | Indicates if EAS ACR is initiated for one of the bundled EAS, then ACR should be initiated for all the EASs belonging to the bundle. The IE may further indicate what actions must be taken if ACR for one or more bundled EAS fails e.g., ACR for all other EAS that are part of the bundle must be cancelled or not. |
| ACID(s) | O | Identifies the AC(s) that can be served by the EAS |
| EAS Provider Identifier | O | The identifier of the ASP that provides the EAS. |
| EAS Type | O | The category or type of EAS (e.g. V2X) |
| EAS description | O | Human-readable description of the EAS |
| EAS Schedule | O | The availability schedule of the EAS (e.g. time windows) |
| EAS Geographical Service Area | O | The geographical service area that the EAS serves. ACs in UEs that are located outside that area shall not be served. |
| EAS Topological Service Area | O | The EAS serves UEs that are connected to the Core Network from one of the cells included in this service area. ACs in UEs that are located outside this area shall not be served. See possible formats in Table 8.2.7-1. |
| EAS Service KPIs | O | Service characteristics provided by EAS, detailed in Table 8.2.5-1 |
| EAS service permission level | O | Level of service permissions e.g. trial, gold-class supported by the EAS |
| EAS Feature(s) | O | Service features e.g. single vs. multi-player gaming service supported by the EAS |
| EAS Service continuity support | O | Indicates if the EAS supports service continuity or not. This IE also indicates which ACR scenarios are supported by the EAS. |
| List of EAS DNAI(s) | O | DNAI(s) associated with the EAS. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [2]. It is a subset of the DNAI(s) associated with the EDN where the EAS resides. |
| List of N6 Traffic Routing requirements | O | The N6 traffic routing information and/or routing profile ID corresponding to each EAS DNAI. |
| EAS Availability Reporting Period | O | The availability reporting period (i.e. heartbeat period) that indicates to the EES how often it needs to check the EAS's availability after a successful registration. |
| EAS Status | O | The status of the EAS (e.g. enabled, disabled, etc.) |

#### 7.26.2.2	Handling of EAS bundle information by the EEC

If AC profile contains an EAS bundle information, the EEC uses it while performing operations on EDGE-1 interface.

#### 7.26.2.3	Handling of EAS bundle information and EAS bundle requirements by the EES

The EES may receive the EAS bundle information from the EEC, EAS or another EES and the EAS bundle requirements from the EAS.

Upon receiving the EAS bundle information and EAS bundle requirements as part of the EAS registration request from the EAS, the EES stores the information and associates the EAS with other EASs providing the same EAS bundle information. The EAS bundle requirements are used by the EES while providing services on EDGE-1, EDGE-3 and EDGE-9 interfaces.

Upon receiving the EAS bundle information as part of the:

-	EAS discovery request from the EEC, EAS or another EES, and depending on the EAS bundle requirements the EES provides in the EAS discovery response information of all the EASs which are part of the EAS bundle.

-	ACR request from the EEC or the EAS, and depending on the EAS bundle requirements the EES takes appropriate ACR related action (e.g., initiate ACR, cancel ACR etc.) for all the EASs which are part of the EAS bundle.

-	Retrieve T-EES request from the S-EES, and depending on the EAS bundle information, the ECS provides in the Retrieve T-EES response information of the T-EES(s) within the same EDN for the bundle EAS.

Enhancements to 3GPP TS 23.558 clause 8.8.4.6	Retrieve EES request

#### 8.8.4.6	Retrieve EES request

Table 8.8.4.6-1 describes the information elements to retrieve T-EES information from the ECS.

Table 8.8.4.6-1: Retrieve EES request

| Information element | Status | Description |
| --- | --- | --- |
| EESID | M | Unique identifier of the EES. |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| EASID | M | The EASID. |
| EAS bundle information | O | EAS bundle which AC requires . |
| Target DNAI | O | The target DNAI information which can be associated with potential T-EES(s) and/or T-EAS(s). |
| UE Identifier | O | The identifier of the UE (i.e. GPSI or identity token) |
| UE location | O | The location information of the UE. The UE location is described in clause 7.3.2. |

### 7.26.3	Solution evaluation

This solution addresses the open issues of KI#18.

To identify the linkages between the EASs, the solution introduces EAS bundle information in the EAS profiles, allowing an EAS to be part of one or more EAS bundles. This information is then used by the ECS and the EES at the time of service provisioning and EAS discovery. To support the queries from the EEC, the EAS bundle information is also added in the AC profiles.

The solution also allows EASs to indicate requirements related to the bundle, such as requesting coordinated EAS discovery or coordinated ACR (considering all scenarios).

The solution also identifies the impacts on several EEL information flows and procedures to handle the EAS bundle information and related requirements.

## 7.27	Solution #27: Enabling AC Association Aware services by selecting common EASs

### 7.27.1	Architecture enhancements

None.

### 7.27.2	Solution description

#### 7.27.2.1	General

This solution addresses key issue #17 on discovery of a common EAS in clause 4.17.

The solution describes enhancements to several edge procedures that enable ACs/EECs of different UEs to share AC association information with the EEL. This AC association is formed such that the services for the associated ACs are optimized by selecting a common EAS.

NOTE:	In the following descriptions within this solution, the term "Associated ACs" is used to denote a set of ACs on different UEs for which the association described above has been configured via an AC association Profile. How the AC association Profile is determined is out of scope of the current specification.

#### 7.27.2.2	New Information Elements

The AC association Profile is proposed as a new IE in TS 23.558:

Table 7.27.2.2-1: AC association Profile

| Information element | Status | Description |
| --- | --- | --- |
| Association ID | M | Identity of the Association |
| AC association type | M | Choice of multi-use or multi-session |
| List of associated UEs characteristics | O | Information for determining the UEs with associated ACs |
| > Associated UE group ID | O | Group ID pre-provisioned (i.e. as External Group ID) to the UEs with ACs in the associated group. If this IE is not present, |
| > Associated UEs service area | O | Service area for determining other UEs in the association. The UE location is described in clause 7.3.2. The optional additional EAS selection criteria describe criteria for the EAS selection (e.g., "same latency for all" or "lowest latency for the own UE location"). |
| List of associated ACs characteristics (NOTE 1) | M | Information for determining the ACs on the associated UEs |
| > common ACID | O | If provided, all ACs in the association have the same AC ID as indicated |
| > common ACs Type | O | If provided, all ACs in the association have the same AC Type as indicated |
| > common ACs Schedule | O | If provided, all ACs in the association have the same AC Schedule as indicated |
| List of common EAS characteristics (NOTE 1) | M | Information for determining the common EAS |
| >Common EAS discovery filter (NOTE 2) | O | Describes the characteristics of a common EASs using the EAS discovery filter described in Table 8.5.3.2-2. |
| > Common EAS aggregate Service KPIs | O | Service characteristics provided by the common EAS, detailed in Table 8.2.5-1. The characteristics are described to meet the requirements for the AC association. |
| NOTE 1: Either "List of associated AC characteristics" or "List of common EAS characteristics" shall be present. NOTE 2: Only the "List of EAS characteristics" IE from Table 8.5.3.2-2 is included. The "List of EAS characteristics" IE must include at least one optional IE, if used as an EAS discovery filter. |  |  |

Editor's note.	The list of IEs in Table 7.27.2.2-1 and whether AC association information should be provided in a new Profile IE, or via existing ones, is FFS.

Editor's note.	Whether the ACs in an AC association information may have different ACIDs is FFS.

#### 7.27.2.3	Enhancements to existing Information Elements

The following enhancements (highlighted with bold text) are proposed to tables in TS 23.558:

Table 7.27.2.3-1: EAS discovery filters

| Information element | Status | Description |
| --- | --- | --- |
| List of AC characteristics (NOTE 1) | O | Describes the ACs for which a matching EAS is needed. |
| > AC profile (NOTE 2) | M | AC profile containing parameters used to determine matching EAS. AC profiles are further described in Table 8.2.2-1. |
| List of EAS characteristics (NOTE 1, NOTE 3) | O | Describes the characteristic of required EASs. |
| > EASID | O | Identifier of the required EAS. |
| > EAS provider identifier | O | Identifier of the required EAS provider |
| > EAS type | O | The category or type of required EAS (e.g. V2X) |
| > EAS schedule | O | Required availability schedule of the EAS (e.g. time windows) |
| > EAS Geographical Service Area | O | Location(s) (e.g. geographical area, route) where the EAS service should be available. |
| > EAS Topological Service Area | O | Topological area (e.g. cell ID, TAI) for which the EAS service should be available. See possible formats in Table 8.2.7-1. |
| > Service continuity support | O | Indicates if the service continuity support is required or not. |
| > Service permission level | O | Required level of service permissions e.g. trial, gold-class |
| > Service feature(s) | O | Required service features e.g. single vs. multi-player gaming service |
| AC association profile | O | Describes an association between ACs and an association between ACs and a common EAS. |
| NOTE 1: Either "List of AC characteristics" ,"List of EAS characteristics" or AC association profile shall be present. NOTE 2: "Preferred ECSP list" IE shall not be present. NOTE 3: The "List of EAS characteristics" IE must include at least one optional IE, if used as an EAS discovery filter. |  |  |

#### 7.27.2.4	Procedure for determining common EES

Editor's note.	The procedure for determining common EES is FFS.

#### 7.27.2.5	Discovery of common EAS Procedure

In this procedure, the EEC sends an EAS discovery request to an EES including AC association Profile. Based the AC association Profile, the EES determines an EAS discovery result that includes a common EAS for the AC association.

Pre-conditions:

1.	An AC association Profile has been provided to the EEC.

Figure 7.27.2.5-1: Common EAS discovery with AC association Profile

1.	The EEC sends an EAS discovery request to the EES. The EAS discovery request includes an AC association Profiles. Included in the AC association Profile is information regarding an association between AC(s) on this UE  and other ACs  indicating that the use of a common EAS is required.

2.	Upon receiving the request, the EES determines whether a common EAS is available to provide services to the associated ACs that meet the criteria specified in the AC association Profile

3.	If the processing of the request was successful, the EES responds to the EEC with an EAS discovery response which includes information about the discovered common EAS able to provide services to the associated ACs.

4.	If the EES is unable to determine a common EAS for the associated ACs, the EES responds with an appropriate failure cause.

#### 7.27.2.6	ACR with common EAS Procedure

Editor's note.	The ACR with common EAS procedure is FFS.

### 7.27.3	Solution evaluation

This clause provides an evaluation of the solution.

Editor's note.	Further investigation is required if this solution is feasible.

## 7.28	Solution #28: Common EAS discovery using EAS selection information

### 7.28.1	Architecture enhancements

None

### 7.28.2	Solution description

#### 7.28.2.1	General

The following solution corresponds to the key issue #17 on Discovery of a common EAS.

#### 7.28.2.2	Procedure

This solution is based on the following principles:

1.	Each EAS instance can be identified uniquely across EDNs.

2.	EAS Selection Information can be stored in the AC profile as the EAS selection information.

3.	For service provisioning, the EEC can include EAS selection information in the AC profile to discover the EES / EDN where a common EAS is available

4.	For EAS discovery, the EEC can include the EAS selection information in the AC profile to discover a common EAS.

This solution assumes that an EAS can only register to a single EES within an EDN, as per Rel-17 version of TS 23.558 [2] clause 6.6.3.3 on reference point cardinality for EDGE-3. Therefore, it is assumed that all UEs discovering the common EAS must do so via the EES where the common EAS is registered. Additionally, requirements such as EEC registration must be followed by the UEs discovering a common EAS.

This solution is compliant with ACID definition as per Rel-17 version of TS 23.558 [2] clause 7.2.5 and allows ACs with the same or different ACID (which belong to applications) to discover and use a common EAS.

Figure 7.28.2.2-1 presents an overview of the procedures for discovery of a common EAS; the scenario represented involves a first UE (e.g. UE-1/AC-1/EEC-1) discovering and using an EAS and a second UE (e.g. UE-2/AC-2/EEC-2) discovering the same EAS used by the first UE-1.

Figure 7.28.2.2-1: Common EAS discovery using EAS selection information

Pre-conditions:

-	The EAS has registered to the EES

1.	The AC-1 registers to EEC-1, and according to Rel-17 procedures defined in TS 23.558, the EEC-1 performs service provisioning with the ECS and performs EAS discovery with the EES. Additionally, the EEC can perform EAS selection declaration if required.

2.	The EEC-1 selects one of the discovered EAS to be used as the common EAS. The EEC-1 uses the EAS Selection Information to form the EAS selection information which can be used to uniquely identify the common EAS within and across EDNs. The EEC-1 can provide the EAS selection information to AC-1.

3.	Not shown on figure 7.28.2.2-1, AC-2 residing on UE-2 can be provided with EAS selection information. How AC-2 obtains the EAS selection information is out of SA6 scope, for example it may be obtained via direct interaction between AC-1 and AC-2 or interaction between AC-2 and the service provider of the EAS.

The AC-2 includes the EAS selection information in its AC profile and then registers with EEC-2, or alternatively updates its existing AC registration with EAS selection information.

NOTE:	If an EAS was discovered prior to receiving EAS selection information, the AC can update its AC profile to trigger step 4 and step 5 to discover the common EAS.

4.	The EEC-2 can perform a service provisioning procedure and includes the AC profile that contains the EAS selection information. The ECS considers the EAS selection information (e.g. the EESID) when selecting the EES and chooses the EES with a matching EESID. The service provisioning response includes the EDN configuration information that contains the EES where the common EAS is registered. Step 4 is optional if the EESID included in the EAS selection information is already known by EEC-2, in such case the EEC-2 can proceed to step 5. Not shown on the figure, the EEC may need to register to the EES if required by service provisioning.

5.	The EEC-2 selects the EES according to the EAS selection information. If the EAS profile of the common EAS is not known at the EEC-2, the EEC-2 can perform the EAS discovery with the selected EES, and includes the AC profile in the EAS discovery request. The EEC-2 performs the EAS selection declaration with the EES; the EES considers if the EEC-2 can use the common EAS. The EAS selection declaration response received at the EEC-2 indicates success/failure; if successful, the AC-2 can access the same common EAS as AC-1.

### 7.28.3	Solution evaluation

The KI #17 has the following open issues:

1)	Whether and how the ACs/EECs of different users can select or be provisioned the same EAS within an EDN?

NOTE:	This open issue is dealing with the issue how different EECs can perform EAS discovery so that they select the same EAS within an EDN, whereas KI#13 is dealing with the issue how, after different EECs have selected different EASs located in different EDNs, these EASs can synchronize their contexts.

2)	Whether and how the ACs/EECs of different users can select or be provisioned a common EAS, even if initially the EECs are communicating with different EDNs?

3)	Whether and how the EEL can support service continuity to ensure that when ACs require the use of service from a common EAS and an ACR operation is needed, ACR operations can be coordinated so that upon completion of the ACR operations the ACs again have services provided by a common EAS.

Overall evaluation of Solution #28:

a)	Enables an AC/EEC to select the same EAS as another EEC within an EDN by extending EAS discovery procedure with EAS selection information.

b)	Enables ACs/EECs of different users to select or be provisioned a common EAS, even if initially the EECs are communicating with different EDNs, by firstly extending service provisioning procedure with EAS selection information, and secondly performing the EAS discovery procedure within that common EDN.

c)	Maintains compatibility with ACR operations by extending pre-existing service provisioning and EAS discovery procedures which are used by ACR procedures.

Editor's note:	It is FFS whether this solution requires detailed procedure updates to show the common EAS selection information.

Editor's note:	This solution requires detailed procedure updates as per bullets a), b) and c) and it is FFS to show the enhancements to existing procedures.

Editor's note:	In the race condition situation (multiple UEs selects different EASs during step 2), how to coordinate common EAS selection is FFS.

## 7.29	Solution #29: Discovery of a common EAS

### 7.29.1	Architecture enhancements

None.

### 7.29.2	Solution description

#### 7.29.2.1	General

Editor's note:	To analyse the feasibility of this solution, further enhancements and clarifications to the description are required.

The following solution addresses open issues 1 and 2 of key issue #17, discovery of a common EAS.

Editor's note:	The solution for open issue 3, support of service continuity, is FFS.

Note:	The communication between AC and AS and between ACs is out of scope of SA6. In the following, some of this communication is only described to enable a better understanding of how the procedures under SA6 responsibility may be embedded in the overall procedure. The description is not intended to exclude scenarios using a different application level communication.

It is assumed that when users want to participate in a real-time multi-user session, their ACs will register with an application server (AS) in the cloud. The AS configures the ACs to form an AC group to which the AS assigns a globally unique Group ID (e.g., a UUID as described in RFC 4122).

During the registration with the AS, users will provide their UE location information to the AS. Dependent on the use case, the location information can be given, e.g., in the format of GPS coordinates, a Cell Identity or a Tracking Area Identity. Furthermore, dependent on the application, the users may be asked to consent to exchanging the location of their UEs with each other. If users consent, then the UE location information is exchanged between the ACs via application level signalling via the AS in the cloud.

Based on the UE location information received from the ACs, the AS determines a Requested EES geographical service area. Dependent on the use case this can be a single coordinate, e.g., the center of mass of the UE/AC locations or the location of a UE/AC taking a specific role in the group (e.g., in a V2X scenario the first truck in platoon), or it can be a geographical area including the current UE/AC locations (without disclosing individual UE/AC locations). The AS provides Group ID, Requested EES geographical service area and, if applicable, the locations of the other ACs to each of the ACs.

Dependent on the application, the distribution of the Group ID and the other parameters by the AS can be triggered by different events, e.g., upon explicit request of a user or when a certain minimum number of users have registered.

Upon receipt of the Group ID, together with the other parameters, each AC requests its EEC to perform EAS discovery.

During service provisioning, the Requested EES geographical service area is received by the ECS and used by it to determine a common EES, by comparing it with the EES geographical service area received from each EES during EES registration.

EAS discovery based on individual UE locations

Later, during EAS discovery, the EEC-1 of user 1 provides the common EES with its own location and with the locations of the other UEs/EECs. Based on the location information of all EECs involved in the session and possibly other information included in the EAS discovery request, e.g., the additional EAS selection criteria, the EES determines a common EAS and indicates it to the EEC-1.

When EEC-2 of user 2 provides the EES with the same set of location information for all the EECs, then the EES will determine the same EAS again.

Dependent on the use case, the EES may apply different additional EAS selection criteria signalled by the EECs to determine the common EAS. E.g., it may be desirable to determine the EAS so that the latency for all the ACs in the session is approximately the same or that the latency for a specific AC is minimized.

EAS discovery based on other information

If users do not consent to share their individual UE location information with other users, then only the Requested EES geographical service area is distributed by the AS to the ACs via application level signalling. Additionally, the AS may send a "group size for EAS discovery" parameter to each AC.

Later, during EAS discovery, each EEC provides the common EES with its own location and optionally, in the Group profile IE, with the Expected Group geographical service area set to the Requested EES geographical service area or with the "group size for EAS discovery" parameter or both.

The EES may initiate determination of a common EAS for the EECs based on different criteria, dependent the received parameters and the application, e.g.:

-	immediately upon receipt of the first EAS discovery request, based on the Expected Group geographical service area, if available;

-	when the number of EAS discovery request is greater or equal to the "group size for EAS discovery" parameter; or

-	a certain time after receipt of the first EAS discovery request,

taking into account the UE location information received from the EECs by this point in time and possibly other information included in the EAS discovery request, e.g., the additional EAS selection criteria.

Editor's note:	Whether the group size should be "minimum group size" is FFS.

Handling of "late-coming" ACs

If a late-coming AC wants to join the multi-user group session after the initial EAS discovery was initiated, it may register with the AS. After registration it will receive from another AC in application layer signalling, e.g., via the AS, the EDN configuration information necessary to connect to the common EES. The details of this signalling are out-of-scope of SA6. The AC can then register with the EES and perform EAS discovery, providing its Group ID. The EES determines the EAS based on the Group ID.

#### 7.29.2.2	Procedure

The following procedure demonstrates the signalling for 2 UEs only, but the same principles can be applied to a larger number of UEs.

Pre-conditions:

1.	ACs have registered with an AS in the cloud to participate in a real-time multi-user session. During the registration, the AS provided the ACs with the Group ID and the Requested EES geographical service area (which may be a single coordinate or an area including the UE locations, see clause 7.29.2.1), and optionally with additional EAS selection criteria for the EES. Dependent on user consent, the AS also forwards the location information of the other UE(s) to each of the ACs. If user consent is not available, the AS may send a "Minimum group size for EAS discovery" parameter to the ACs.

Figure 7.29.2.2-1: Discovery of a common EAS

1.-2.	Each AC requests its EEC to perform EAS discovery. The request includes the AC profile, the Group ID, the Requested EES geographical service area, and optionally the location information of the other UEs involved in the multi-user session in the request and the additional EAS selection criteria for the EES.

3.-4.	EEC1 and EEC2 initiate service provisioning, including the AC profile, the Group ID, the Group profile IE including the Requested EES geographical service area, and optionally a Group KPI IE.

Editor's note:	Whether the Group profile IE and Group KPI IE are required is FFS

5.-6.	Based on the AC profile, the Requested EES geographical service area and the other information provided by the EEC, the ECS determines a suitable common EES and includes it in the service provisioning response. If more than one suitable EES is available, the ECS can use additional information as input parameter to determine a unique EES.

Note 1:	For example, the ECSs can use the Group ID and an algorithm shared between the ECSs to derive the common EES from the list of available EESs.

If the ECS wants to provision the EECs with additional EESs, e.g., for service continuity, then the common EES is included as first EES in the first entry of the list of EDN configuration information.

7.-8.	EEC1 and EEC2 initiate EAS discovery towards the common EES, including the AC profile, the Group ID, the Group profile IE which optionally includes the requested EES geographical service area (set to the Requested EES geographical service area) or the location information of all UEs involved in the multi-user session, the additional EAS selection criteria for the EES, and a Group KPI IE.

9.-10.	The EES initiates determination of a common EAS for the EECs based on different criteria, dependent the received parameters and the application, e.g.:

i)	immediately upon receipt of the first EAS discovery request, based on the Expected Group geographical service area or the location information of all UEs involved in the multi-user session, if received;

ii)	when the number of EAS discovery request is greater or equal to the "group size for EAS discovery" parameter; or

iii)	a pre-defined time after receipt of the first EAS discovery request,

taking into account the Expected Group geographical service area or the UE location information received by this point in time and possibly other information included in the EAS discovery request, e.g., the additional EAS selection criteria.

Note 2:	The EES can also utilize the capabilities of the 3GPP core network to determine a UE location.

The EES includes the determined common EAS, e.g. EAS-1(A), in the EAS discovery response.

### 7.29.3	Solution evaluation

This clause provides an evaluation of the solution.

## 7.30	Solution #30: Common EAS selection

### 7.30.1	Architecture enhancements

Figure 7.30.1-1: EDGEAPP architecture enhanced with binding server

The EDGE-X reference point is introduced to support EES interaction with binding server. The EES can store, update and remove the binding information via EDGE-X reference point.

The binding server is deployed in a central cloud and it is a single server maintaining binding information.

Editor's note:	Whether binding server can be deployed in EDN is FFS.

NOTE:	CBS can be co-located with an ECS in a deployment.

Architecture enhancements in clause 6.7 is the basis for this solution.

### 7.30.2	Solution description

This solution addresses KI#17.

The UEs can be grouped together to consume EAS services on the same EAS endpoint.

The group information (e.g. group id) can be used as part of the binding information to support anchor those UEs to the same EAS. The binding information are maintained on a Binding Server (BS). When an EES is aware of the selected EAS (e.g. via EEC sent EAS selection declaration), the BS is contacted by the EES and the BS can decide whether to proceed the currently selected EAS or instruct to use another EAS (which already has other established session(s) for the group).

The EDN id is used to identify EDN and is also part of the binding information. Figure 7.30.2-1 show the detailed procedure with a Central Binding Server (CBS) for common EAS pre-selection in EES.

In the following figures, ACs with same AC ID on different UEs is shown as example, it is also possible that ACs with different AC IDs supplied by the same AC provider (e.g. AC developed for iphone and AC developed for Android phone) need to communicate with each other via EAS.

Pre-conditions:

1.	The EEC is aware of the group info from AC via EDGE-5 reference point.

Figure 7.30.2-1: select a common EAS for UEs, common EAS pre-selection in EES

In EDN1:

1.	During EAS discovery, EEC 1 sends EAS discovery request to EES 1 including EAS ID, UE ID (of UE1), AC ID (of AC1) and group info (e.g. group id x).

2.	The EES 1 pre-selects an EAS and sends group binding request message to CBS with the received information from EEC 1 and in addition the EDN id (of EDN 1).

3.	There is no existing binding information found so that the CBS creates a new binding information for the UE group.

4.	The CBS responds group binding request with a result indicating OK to proceed with the requested EAS.

5-7.	With the received EAS information, the EES 1 responds the EAS discovery request with the selected EAS to the EEC 1 and consequently AC1 connects to the selected EAS after being informed by EEC 1.

In the same EDN (i.e. EDN1):

8.	During EAS discovery, EEC 2 sends EAS discovery request to EES 2 with the selected EAS information including EAS ID, UE ID (of UE2), AC ID (of AC1) and group info (e.g. group id x).

9.	The EES 2 pre-selects an EAS and sends group binding request message to CBS with the received information from EEC 2 and in addition the EDN id (of EDN 1).

10.	The CBS finds existing binding information for the UE group based on group info.

11.	The CBS responds with a result indicating recommendation to use common EAS (including the associated EES info) which is EES 1 selected EAS.

12-14.	With the received EAS information, the EES 2 responds the EAS selection to the EEC 2 and consequently AC1 connects to the common EAS. EEC 2 may contact EES 1 for subsequent procedures (e.g. EEC registration, ACR subscription).

Editor's note:	Current solution describes that EAS determined by the 1st UE's activity in the order of time is used as common EAS. Whether extra information is needed to ensure that all UE members will get a good and fair performance is FFS.

Editor's note:	Binding update and removal procedure is FFS.

Editor's note:	Common EAS pre-selection in UE and common EAS relocation are FFS.

### 7.30.3	Solution evaluation

This clause provides an evaluation of the solution.

Editor's note:	Further investigation is required if this solution is feasible.

## 7.31	Solution #31: Discover common EAS

### 7.31.1	Architecture enhancements

None.

### 7.31.2	Solution description

#### 7.31.2.1	General

The following solution addresses open issues of key issue #17, Discovery of a common EAS.

As specified in TS 23.558 (Rel-17), the EEC of one user may select EAS which is different from the EAS selected by another user although both users are part of the same communication session. If different EASs are selected for the users of the same communication session, then different EASs need to synchronize with each other in order to maintain the communication session.

For certain use cases involving real-time communication in a multi-user session, both between AC and EAS and between different ACs via the EAS, it may be necessary or beneficial to use services from a single common EAS to meet the strict latency requirements and to avoid the need for inter-EAS synchronization.

Dependent on the use case, the EEL may apply different additional criteria to determine this common EAS. E.g., it could be desirable to determine the EAS so that the latency for all the ACs in the session is approximately the same or that the latency for a specific AC is minimized.

#### 7.31.2.2	Procedure

The following procedure demonstrates the Discovery of a common EAS.

The procedure supports selection of a common EAS:

-	for the same ACID on UEs of different Users (aware or unaware of each other), within the same EDN

-	for group of ACIDs on UEs of same User within the same EDN

-	per group within the same EDN and for the same ACID(s)

-	for new UEs requesting to join after a common EAS is already selected.

Pre-conditions:

1.	AC Profile is enhanced with Grouping required information that tells whether the AC requires a multi-user session or multi-AC session etc.

2.	EAS registers to EES according to clause 8.4.3 EAS Registration in TS 23.558.

Figure 7.31.2.2-1: Discover common EAS

1.	EEC-1 received a trigger to initiate EAS discovery.

2.	EEC-1 performs the EAS discovery, as described in TS 23.558 V17.3.0, where an enhanced AC profile including Grouping required information is included in the request towards EES-1. Grouping required can be for a multi-user session or multi-AC session or a combination of both. Group ID IE (which indicates the ID for grouping of Users of same AC or grouping of ACs.) is present only when the expectation is to enable multiple groups while the UEs are connected to EDN for the same EASID.

3.	EES-1 considers Grouping required information as one of the discovery filters to decide the list of EASes to return in the response to EEC-1's EAS discovery request. The EES-1 checks if there is already a selected EAS corresponding to the received Grouping required information from EEC-1. If it exists then the EES-1 returns the selected EAS. Otherwise, the EES-1 determines the list of EAS as specified in TS 23.558 (Rel-17).

4.	When multiple EASs are discovered for a specific AC, the EEC-1 may select one or more EASs to enable AC communication with one of the selected EASs i.e. the EEC-1 (or AC and EEC-1) selects the initial EAS from the discovered EAS candidates, considering the Grouping required information.

5.	EEC-1 sends a Selected EAS announcement request with AC ID, EASID, EAS endpoint, and UE ID to the selected EES-1 (which is determined based on the selected EAS). The request shall include Group ID IE if this information is present in Step-2. The mapping of selected common EAS with grouping required information and Group ID (if present) is stored at EES-1.

6.	EEC-1 is then responded to by the selected EES-1 with the success/failure of the request.

7.	EES-1 determines which other EESes to be informed about the selected EAS e.g. serving the same AC within the EDN as per the procedure in clause 8.8.3.3 of TS 23.558 for the EES-1 to retrieve the EES-n information from the ECS.7.1 Selected EES-1 contacts ECS along with EASID information of the Selected EAS to determine which other EES(s) serve the same EASID.

7.2 ECS provides endpoint information of other EES(s) as described in table 8.3.3.3.3-2, corresponding to the requested EASID information.

8.	Selected EES-1 then declares common EAS selection to all the determined EESes (EES-n) along with the Grouping required information and Group ID (if present).

9.	EES-n stores the received common EAS information along with the Grouping required information and Group ID (if present) and sends back an acknowledgement to EES-1.

10.	Similar to step-2, EEC-2 performs the EAS discovery, as described in TS 23.558 V17.3.0, where an enhanced AC profile including Grouping required information and optionally Group ID is included in the request towards EES-n.

Editor's note:	Handling of race conditions e.g. EAS discovery before announcement in Step 8, is FFS.

11.	EES-n checks if there is already a selected EAS corresponding to the received Grouping required information from EEC-2.

12.	If a common EAS is available, EES-n then informs the selected EAS to the EEC-2. When Group ID is present in Step-10, EES-n checks if common EAS is selected corresponding to the Group ID and inform the selected common EAS to the EEC-2.  Otherwise, Step 3 to Step 9 are performed to select and announce a common EAS.

The EDN configuration information received from ECS may then be used by EEC-1 and EEC-2 for establishing a connection to the common EAS.

Enhancements to 3GPP TS 23.558 Table 8.2.2-1 AC Profile

Table 8.2.2-1: AC Profile (enhanced)

| Information element | Status | Description |
| --- | --- | --- |
| ACID | M | Identity of the AC. |
| AC Type | O | The category or type of AC (e.g. V2X). This is an implementation specific value. |
| Grouping required | O | Indicates Grouping is required for the application. |
| > multi-user session (NOTE 1) | O | Indicates application requires multi-user session |
| > multi-AC session (NOTE 1) | O | Indicates multiple ACs would be part of the session |
| >> List of ACIDs (NOTE 2) | O | Indicates one or more ACIDs to be part of the session |
| Preferred ECSP list | O | When used in a service provisioning request, this IE indicates to the ECS which ECSPs are preferred for the AC. The ECS may use this information in the selection of EESs. |
| AC Schedule | O | The expected operation schedule of the AC (e.g. time windows) |
| Expected AC Geographical Service Area | O | The expected location(s) (e.g. route) of the hosting UE during the AC's operation schedule. This geographic information can express a geographic point, polygon, route, signalling map, or waypoint set. |
| AC Service Continuity Support | O | Indicates if service continuity support is required or not for the application. The IE also indicates which ACR scenarios are supported by the AC and which of these are preferred by the AC. |
| List of EASs | O | List of EAS that serve the AC along with the service KPIs required by the AC |
| > EASID | M | Identifier of the EAS |
| > Expected AC Service KPIs | O | KPIs expected in order for ACs to receive currently required services from the EAS, as described in Table 8.2.3-1 |
| > Minimum required AC Service KPIs | O | Minimum KPIs required in order for ACs to receive meaningful services from the EAS, as described in Table 8.2.3-1 |
| NOTE 1: Either one of these or both are present depending on the application scenario. NOTE 2: Atleast one ACID is mandatory when multi-AC session IE is present. |  |  |

Editor's note:	Using list of ACIDs for EAS discovery is FFS

### 7.31.3	Solution evaluation

This clause provides an evaluation of the solution.

Editor's note:	Further investigation is required if this solution is feasible.

## 7.32	Solution #32: Dynamic EAS instantiation triggering and notification

### 7.32.1	Architecture enhancements

None.

### 7.32.2	Solution description

#### 7.32.2.1	General

This solution addresses the Key issue #9: Enhancement of dynamic EAS instantiation triggering for efficient utilization of EDN resources for EAS deployment.

As specified in TS 23.558 (Rel-17), EES may trigger the EAS instantiation dynamically if there is no instantiated EAS that matches the requesting service characteristics during EAS discovery.

By collecting one or more of these triggering input events, EES may determine if there is a need for EAS instantiation based on the pre-configured information about instantiable EASs with further considering the requesting service characteristics (e.g., location, latency) by EEC or service load/capacity (e.g., number of service sessions) of EAS. If such a need for EAS instantiation determined, EES may sends a report for a need of the EAS instantiation to the ECSP management system (which is specified in TS 28.538 [x]) to consider instantiating the target EAS that is determined to instantiate by invoking an MnS API of the ECSP management system.

When the target EAS has been instantiated, the EES may obtain the EAS profile based on the updated configuration information by the ECSP management system. Then EES may further notify the instantiation result of the target EAS with the EAS profile to the corresponding EECs in order to inform the EECs of the availability of the EAS instance.

In this solution, the EAS discovery subscribe-notify procedures are re-used as specified in the clause of 8.5.2.3 of TS 23.558 [2]. EEC may request EES for subscribing to EAS discovery. When the EES gets informed of the EAS instantiation, the EES notifies the EECs whose EAS discovery filters match with the EAS instance.

NOTE 1:	How and when a triggering is determined by EES is upon implementation and out the scope of this solution.

NOTE 2:	The pre-configured information about instantiable EASs may be provided by the ECSP management system but such a mechanism is out of the SA6 scope.

NOTE 3:	With the EAS instantiation request, EES just provides its indication for demanding instantiation of the target EAS to the ECSP management system. Any decision or further actions to the requests are up to the ECSP management system.

NOTE 4:	The MnS APIs for EAS instantiation request/notification should be provided by the ECSP management system and should be consulted with SA5.

The solution can be summarized as follows:

Dynamic EAS instantiation triggering and notification

a.	Triggering inputs events:

i.	No available EAS instances matched during EAS discovery

b.	Triggering determination:

i.	Up to EES implementation

ii.	Based on the triggering input events and pre-configured information about instantiable EASs

iii.	Further considerations:

-	EEC's requesting service characteristics (e.g., location, latency) as specified in EAS discovery filters

-	EAS's service load/capacity (e.g., number of service sessions) maintained by EES

c.	Triggering actions

i.	Invoking an MnS API of the ECSP management system for reporting a need of instantiation of a target EAS

d.	Post-triggering actions

i.	Receiving a notification for instantiation result from the ECSP management system by configuration update

ii.	Notifying the EAS instantiation to the corresponding EECs which have initiated the triggering input events for EAS availability change.

Editor's note:	Further considerations for the other EAS discovery events (e.g., T-EAS discovery by S-EES and S-EAS; or EAS Service API discovery as studied in KI#2) are FFS.

#### 7.32.2.2	Dynamic EAS instantiation triggering and notification procedures

The Figure 7.32.2.2-1 depicts the essential operational steps for dynamic EAS instantiation triggering and notification procedures.

Pre-conditions:

1.	EES is pre-configured with the information about instantiable EASs which may be provided by the ECSP management system.

Figure 7.32.2.2-1: Dynamic EAS instantiation triggering and notification procedures

1a-1b.	EECs proceed EAS discovery subscription request with EES, which are triggering input events for dynamic EAS instantiations.

2a-2b.	EES sends an EAS discovery subscription response to the EEC.

3.	Based on the triggering input events, the EES determines if there is a need for EAS instantiation considering the requesting service characteristics provided by EAS discovery filters; or service load/capacity (e.g., number of service sessions) of EAS maintained by the EES.

4.	If such a need for EAS instantiation determined, EES sends a report for a need of the EAS instantiation to the ECSP management system for considerations to instantiate the target EAS by invoking an MnS API of the ECSP management system.

5.	The MnS of the ECSP management system responds to the EES with indicating that the requesting EAS instantiation will be considered.

6.	The ECSP management system determines if the requested EAS instantiation is acceptable by analyzing the deployment requirements and available resources; and proceeds the corresponding actions. (out the scope of this specification)

7. 	If the target EAS is instantiated successfully, the EES gets informed by an EAS registration procedure of the instantiated EAS with the EAS information (e.g., EAS profile).

8a-8b.	The EES notifies the EAS instantiation result with EAS discovery notification including the EAS profile to the relevant EECs which have already initiated the triggered input events for the target EAS instance.

### 7.32.3	Solution evaluation

This solution allows for an EES to dynamically trigger the EAS instantiation using 1) indication of a need for EAS instantiation to the ECSP management system; and 2) the EAS discovery notification for informing EEC of the result of the EAS instantiation with the EAS profile.

This solution addresses the key issue #9: enhancement of dynamic EAS instantiation triggering as specified in the clause 4.9.

This solution relies on the EDGEAPP architecture as specified in TS 23.558 [2] with extended capabilities of EES to determine if there is a need for EAS instantiation considering the requesting service characteristics by EEC or service load/capacity of EAS.

This solution also relies on a new MnS API for indication of a need for EAS instantiation, which shall be provided by the ECSP management system by consulting with SA5.

Editor's Note: Whether a new MnS API is needed needs to be consulted with SA5.

## 7.33	Solution #33: Support for EEC Discovery of EAS(es) before instantiation

### 7.33.1	Architecture enhancements

None.

### 7.33.2	Solution description

#### 7.33.2.1	General

The following solution corresponds to key issue #9 on enhancements of dynamic EAS instantiation triggering. This solution helps ensure that the proper number of EAS instances are instantiated in the EDN and the solution addresses what information may be utilized by an EES to decide to trigger dynamic EAS instantiation.

In this solution, the EES is configured by the Management Service (MnS) with information on EAS(s) that can be dynamically instantiated in an EDN; this list provides the information elements needed for requesting EAS instantiation.

This solution updates the discovery procedure so that EEC can discover EAS(es) executable in an EDN, wether they are instantiated or not. The solution then introduces a new EAS selection procedure so that the EEC can indicate to the EES that it wants to access the services of an EAS that is not yet instantiated. The benefit of this procedure is that EAS instantiation happens after an EEC has selected an EAS, and only at the EES associated with the selected EAS.The solution dissociates EAS discovery, which can be performed with multiple EESes, or can have been performed in a distant past (e.g. relies on caching), from the EAS selection performed at the EEC by providing information elements to the EES about the selection and imminent use of an EAS.

Editor's note:	It is FFS how to solve the issue of failure of EAS instantiation after EAS selection.

Editor's note:	Whether to merge step 6 to step 4 to allow EEC provide intention of selection in discovery request to avoid issues caused by selection of uninstantiated EAS is FFS.

#### 7.33.2.2	Procedure

This procedure presents a high-level overview of Solution #33.

Figure 7.33.2.2-1: High-level overview of solution #33

1.	On initial deployment, the Management Service (MnS) is requested to perform EES deployment and can additionally be requested to perform initial EAS deployment. Details on requesting EES and EAS instantiation is specified in 3GPP TS 28.538 [20].

To support the dynamic EAS instantiation functionality, the EES needs a list of EAS that it is allowed to dynamically instantiate and corresponding EAS information elements needed for requesting EAS instantiation.

Editor's note:	How the dynamic EAS instantiation information is made available to the EES is in the scope of SA5.

2.	EES registers to the ECS following the procedures described in 3GPP TS 23.558 [2] clause 8.4.4. The EES profile may include EAS that the EES is allowed to instantiate.

3.	EEC performs the service provisioning following the procedures described in 3GPP TS 23.558 [2] clause 8.3.3.

4.	When the AC requests application server access, the EEC can perform EAS discovery with one or more EES, or alternatively the EEC can use cached EAS information following the procedures described in 3GPP TS 23.558 [2] clause 8.5. In this solution, the EAS discovery response may include information about EAS(s) that are not currently instantiated (e.g. EAS discovery response may be updated with Instantiable EAS List (O) information element or EAS Endpoint may be optional in EAS Profile to infer uninstantiated EAS).

5.	The EEC performs EAS selection and can select an EAS that is not currently instantiated.

6.	If the EEC selects an EAS that is not currently instantiated, then the EEC informs the EES of the selected EAS by sending an EAS selection request to the EES. Otherwise, the procedure continues to step 10.

EAS selection request is a new message that contains the following information elements: Requestor identifier (M), UE identifier (O), Security credentials (M), Selected EAS ID (M).

Editor's note:	Whether a new EAS selection API is needed or existing API (e.g. such as EEC registration) can be re-used to indicate selected EAS is FFS.

7.	The EAS selection request triggers the EES to verify if the instantiation of the selected EAS is required; since the selected EAS is not instantiated, the EES requests the MnS to perform EAS instantiation for the selected EAS. Upon reception of the EAS instantiation request, MnS verifies EAS instantiation requirements and performs EAS instantiation as specified in 3GPP TS 28.538 [20].

Upon instantiation of the EAS by the MnS, the EES is informed by MnS of the result of EAS instantiation.

NOTE:	The instantiated EAS can be discovered by other EECs and used by other ACs.

8.	Step 7 happens only if EAS instantiation triggered in step 6 was successful. At start-up, newly instantiated EAS registers to the EES following procedures described in 3GPP TS 23.558 [2] clause 8.4.3.

9.	Using the EAS instantiation result received from the MnS in step 6 and the registration information provided at EAS registration in step 7; the EES sends the EAS selection response to the EEC to provide EAS connectivity information.

EAS selection response is a new message that contains the following information elements: Response Status (M), EAS Profile (O), Failure cause (O).

10.	The AC accesses the selected EAS.

### 7.33.3	Solution evaluation

This clause provides an evaluation of the solution.

## 7.34	Solution #34: EDGE-5 APIs

### 7.34.1	Architecture enhancements

None.

### 7.34.2	Solution description

#### 7.34.2.1	General

This solution corresponds to key issue #4 on EDGE-5 reference point between ACs and the EEC.

The solution provides an API for the AC to request for EEC's services for edge enablement corresponding to EEC's capabilities. Using this API, AC may request the EEC for EEL services. The solution defines APIs for one-time request/response operations for EAS discovery and ACR operations. Additionally, The AC can request an AC subscription. The EEC creates the subscription and when required, performs necessary operations such as EAS discovery, ACR etc., delivering notifications to the AC as required.

NOTE:	EEC can initiate any EDGE-1 or EDGE-4 operation without receiving a request or without receiving AC related information from the AC.

#### 7.34.2.1	Procedure

##### 7.34.2.2.1	General

The solution provides the following procedures:

-	AC registration request;

-	EAS discovery request;

-	ACR request;

-	AC subscription; and

-	AC notification.

NOTE:	Details on how the AC and EEC communicate with each other is out of scope.

Editor's note:	Whether a general AC subscription-notification to all EEC services or individual subscriptions for each EEC service or both is required FFS.

##### 7.34.2.2.2	AC registration request

Pre-conditions:

1.	The AC can communicate with the EEC.

NOTE 1:	Details on how the AC and EEC communicate with each other is out of scope.

Figure 7.34.2.2.2-1: AC registration request procedure

1.	The AC sends an AC registration request to the EEC. The request includes the AC profile, AC's security credentials and optionally the EAS characteristics and EAS discovery filters. The request may also include a list of EEC's services that AC requires the EEC to handle. The request additionally includes ECS configuration information if the AC is edge-aware and configured with the ECS configuration information.

NOTE 2:	The ASP providing the AC and the ECSP providing the ECS can have edge computing service provider service agreement as in clause 9. The ECS configuration information configured in the AC is based on the service agreement.

2.	The EEC checks AC's security credentials and validates the request.

3.	If the request is successfully validated, the EEC registers the information provided in the request and responds back to the AC with AC registration response. The AC registration response includes the list of capabilities supported by the EEC e.g., which service continuity scenarios are supported by the EEC. If the request in step 1 included a list of EEC's services AC requires from the EEC, the response also includes a list of EEC's services that AC is authorized for.

NOTE 3:	The mechanisms used for authentication and authorization between AC and EEC is out of scope of this specification. EEC can use local policies, user preferences, ASP services agreement(s) (see clause 9) to authorize the request from the AC.

NOTE 4:	Additional procedures between AC and EEC to update the registration or deregister are necessary.

NOTE 5:	When the ECS configuration information is provided from an AC, the EEC can use the ECS configuration for initial service provisioning for the AC that provided the ECS configuration information if there is no ECS configuration information is provided from the 5GC.

##### 7.34.2.2.3	EAS discovery request

Pre-conditions:

1.	The AC can communicate with the EEC.

Figure 7.34.2.2.3-1: EAS discovery request procedure

1.	The AC sends an EAS discovery request to the EEC. The request includes AC's security credentials and may include AC profile and EAS discovery filters.

2.	The EEC checks AC's security credentials and validates the request.

3.	If the request is successfully validated, the EEC determines if the required EAS is available or not. The EEC may use information cached or preconfigured at the EEC or may use the EAS discovery procedures to query the EES. If step 1 includes the AC profile or EAS discovery filters, then the EEC may utilize the provided AC profile and filters, to form the EAS discovery request towards EES. If step 1 does not include AC profile and EAS discovery filters, and AC registration was performed, the EEC may utilize the AC profile provided by the AC during AC registration. The EEC also needs to take user privacy requirements, e.g., regarding the disclosure of location information towards the network into account. If required, e.g., when EAS discovery procedures returns a list of EASs, the EEC performs EAS selection based on the information received in step 1 and the AC profile. The EEC can perform EAS discovery with different EESs before selecting an EAS.

NOTE 1:	SA3 recommendations, if any, on how the user or the AC can consent, e.g., to the disclosure of location information and the use of the AC ID in the signalling towards the network must be considered during normative.

NOTE 2:	If required, the EEC can perform service provisioning procedure, or EEC registration procedure or both, before performing the EAS discovery procedures. EEC may already have captured EESs and EASs availability for present location; so that the AC's request (step #1) can be replied to quickly and efficiently.

NOTE 3:	The EEC can include AC profiles of more than one AC in the EAS discovery request sent to the EES.

4.	The EEC responds back to the AC with the EAS discovery response. The response includes the EAS profile(s) of the available EAS(s).

##### 7.34.2.2.4	ACR request

Pre-conditions:

1.	The AC can communicate with the EEC.

Figure 7.34.2.2.4-1: ACR request procedure

1.	The AC sends an ACR request to the EEC. The request includes AC's security credentials, type of requested operation (i.e., ACR detection, ACR initiation) and AC profile. If the request is to initiate the ACR, the request may also include the target EAS information.

2.	The EEC checks AC's security credentials and validates the request.

3.	If the request is successfully validated, the EEC process the request from the AC. If the type of requested operation in the request received in step 1 is:

-	ACR detection, then the EEC determines if ACR is required or not. If it is required, the EEC uses one of the EEC initiated ACR scenarios or launches ACR with action "determination", leading to S-EES executed ACR;

-	ACR initiation, then the EEC uses one of the EEC initiated ACR scenarios and initiate ACR. If the request in step 1 also includes target information, the EEC uses it to select the ACR targets;

##### 7.34.2.2.5	AC subscription request

Pre-conditions:

1.	The AC can communicate with the EEC.

Figure 7.34.2.2.5-1: AC subscription request procedure

1.	The AC sends an AC subscription request to the EEC. The request includes AC's security credentials, a list of EEC's services that AC requires the EEC to handle, and related parameters such as AC profile. If the subscription request includes:

-	EAS discovery or EAS dynamic information subscription, then the request may include EAS discovery filters

-	ACR management, then the request may include type of ACR operations:

-	ACR monitoring, where the EEC monitors the need for ACR and notifies the AC as and when required e.g., on receiving ACR related notifications on EDGE-1 interface.

-	EEC managed ACR, where the EEC monitors the need for ACR. If need for ACR is detected, then the EEC decides and initiates ACR using one of the EEC initiated ACR scenarios. The EEC notifies the AC about the imminent ACR and may include the target information.

2.	The EEC checks AC's security credentials and validates the request.

3.	If the request is successfully validated, the EEC creates the subscription and sends an AC subscription response message to the AC. The response includes the list of services that the EEC will handle and related details.

4.	The EEC executes the services e.g., EAS discovery, ACR management, and notifies the AC with information as necessary. The EEC may use locally cached information or configurations while providing services to the AC.

NOTE:	Additional procedures between AC and EEC to update the subscription or to unsubscribe are necessary.

##### 7.34.2.2.6	AC notification

Pre-conditions:

1.	The AC has subscribed to the EEC.

Figure 7.34.2.2.6-1: EAS discovery notification procedure

1.	An event occurs at the EEC that satisfies trigger conditions for notifying a AC e.g., EEC detects a need for Application Context Relocation.

2.	The EEC sends an AC notification to the AC with relevant information related to the event triggered in step 1.

### 7.34.3	Solution evaluation

This solution relates to KI#4 on EDGE-5 interface. The solution provides multiple procedures to allow uniform interaction of ACs with the EECs. The solution specifies procedures for AC registration, EAS discovery requests/response, ACR related request/response and AC's subscription to EEC.

The solution has no impacts on Rel-17 cardinalities defined for EDGE-5. Aspects related to mutual authentication, authorization and user consent require inputs from SA3.

## 7.35	Solution #35: EEC selected ACR scenarios

### 7.35.1	Architecture enhancements

None

### 7.35.2	Solution description

#### 7.35.2.1	General

The following solution corresponds to the key issue #19 on ACR scenario combination.

#### 7.35.2.2	Procedure

In this solution, the EEC is responsible to determine the ACR scenario(s) that should be used for an AC and EAS pair. The EEC performs ACR selection at EAS selection time.

The following ACR selection outcomes are possible:

-	The EEC can select no ACR scenario which indicates that service continuity is not supported or needed.

-	The EEC can select one ACR scenario which indicates that service continuity is supported and needed, but that ACR coordination is not needed.

-	The EEC can select two or more ACR scenarios which indicates that service continuity is supported and needed, and that ACR coordination is needed.

ACR coordination ensures that only one of the selected ACR scenarios proceeds through the ACR execution phase. Coordination of concurrent ACR execution is out of scope of this solution.

Figure 7.35.2.2-1 presents an overview of the procedures for ACR selection at the EEC.

Figure 7.35.2.2-1 EEC selected ACR scenarios

Pre-condition:

-	EEC has performed service provisioning procedure with the ECS.

1.	The EEC performs EAS discovery using procedures defined in 3GPP TS 23.558 [2] clause 8.5 to obtain a list of discovered EAS(s).

The discovered EAS list contains EAS(s) that are compatible with the AC and EEC ACR capabilities provided in the EAS discovery request or subscription.

2.	The EEC (or AC and EEC) selects one EAS from the discovered EAS list.

Additionally, the EEC selects the ACR scenario(s) that should be used for the given AC and selected EAS. ACR selection can result in zero or more ACR scenarios being selected. For selecting the ACR, the EEC minimally considers the ACR capabilities of the AC from AC Profile, of the EEC (known locally), of the EES from service provisioning response received from the ECS (pre-requisite), and of the EAS from EAS Profile and obtained at EAS discovery.

NOTE:	ACR selection criteria minimally can include ACR capabilities of the AC/EEC/EES/EAS; other criterias are possible and left out of scope of the current solution.

Editor's note:	Whether AC service KPI also will be considered by the EEC to decide on the ACR scenario(s) is FFS.

3.	The EES uses the list of selected ACR scenarios to determine if it should perform ACR detection and/or ACR decision.

4.	The EES sends an ACR selection notification to the selected EAS providing the selected ACR scenario list. Not shown on the figure, the EAS previously subscribed to receive ACR notifications. The EAS uses the selected ACR scenarios list to determine if it should perform ACR detection and/or ACR decision.

5.	The EES sends the selected EAS declaration response to the EEC indicating success or failure of the EAS announcement request.

### 7.35.3	Solution evaluation

This clause provides an evaluation of the solution.

## 7.36	Solution #36: Alignment of EDGEAPP and ETSI MEC

### 7.36.1	Architecture enhancements

Architecture requirements in clause 5.4 will serve as the guiding principles for the alignment of EDGEAPP and ETSI MEC architectures.

The following requirements will serve as the guiding principles for the alignment of EDGEAPP and ETSI MEC architectures.

1.	The scope of 3GPP alignment efforts between EDGEAPP and ETSI MEC shall be limited to architecture enhancements that apply only to EDGEAPP.

NOTE: 3GPP will liaise with ETSI ISG MEC for architecture recommendation (if any).

2.	The architecture enhancements to support alignment shall ensure backwards compatibility with the existing EDGEAPP architecture.

3.	The architecture enhancements shall not consider to align features exclusive to the each of the EDGEAPP and ETSI MEC architectures, i.e. the architecture enhancements shall only focus on the overlapping aspects between the EDGEAPP and ETSI MEC architectures.  The alignment aspects in the present release of this specification includes the following:

a.	alignment of EAS profile (EDGEAPP) and appInfo (ETSI MEC),

b.	alignment of EDGE-3/Mp1 reference points

c.	alignment of EDGE-9/Mp3 reference points

d.	usage of CAPIF between the two architectures

NOTE: The term alignment does not imply that the reference points will be exact equivalents.

Editor's note:	Any additional aspects to be considered for alignment is FFS.

4.	The architecture enhancements shall ensure that EDGEAPP architecture can remain as a standalone or a complete system i.e. EDGEAPP can be deployed independent of the ETSI MEC architecture.

5.	The architecture enhancements shall analyse impacts to other working groups e.g. SA2, SA3 and SA5.

### 7.36.2	Solution description

#### 7.36.2.1	General

This solution is to address KI# 5: Alignment of EDGEAPP and ETSI MEC.

In particular, the solution addresses the third open issue, which is providing solutions based on the observations, study, and comparison of ETSI MEC and EDGEAPP specifications.

#### 7.36.2.2	Alignment of EAS registration and MEC application registration

According to Annex C of 3GPP TS 23.558 [2] MEC application and MEC platform in ETSI MEC correspond respectively to EAS and EES in 3GPP specification. Similarly, Mp1 interface corresponds to EDGE-3. According to observation A.3-1 in Annex A, registration of an instance of an application is supported by both ETSI MEC and 3GPP. Additinally, observation A.3-2 further clarifies that some of the IEs of the EAS profile may overlap with the ones defined in AppInfo, while some other IEs are specific to each of the EAS profile or AppInfo.

In order to support registration of an application on EES it is required that the registration request includes at least the mandatory IEs that are required for EAS registration, i.e. EAS ID and EAS endpoint. On the other hand, according to ETSI GS MEC 011 [14] the application registration request must include AppName and App Provider.

AppInfo which is defined in ETSI GS MEC 011 [14] includes AppName and App Provider as mandatory IEs and endpoint as an optional IE. AppName can be considered equivalent to EAS ID. Similarly, App Provider corresponds to EAS Provider Identifier. Endpoint in ETSI GS MEC 011 [14] can be directly mapped to EAS endpoint. See Table 7.36.2.2-1 for a comparison between the EAS profile and appInfo.

Table 7.36.2.2-1: Comparison between EAS Profile and appInfo

| IE in EAS profile | Status | IE in appInfo [14] | Status [14] | Remarks |
| --- | --- | --- | --- | --- |
| EASID | M | appName | M | 3GPP specification [2]: The EASID identifies a particular application for e.g. SA6Video, SA6Game etc. For example, all Edge SA6Video Servers will share the same EASID. NOTE: The definition of the EASID is out of scope of this specification. ETSI MEC specification [14]: Name of the application. It shall be consistent with the appName in the AppD, if an AppD is available. In AppD, it is defined as: Name to identify the MEC application. In both specifications, the IE is used to identify a particular application. The purpose of the IE is the same in both specifications, and can be considered equivalent. |
| EAS Endpoint | M | endpoint | O | 3GPP specification [2]: Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EAS. ETSI MEC specification [14]: Endpoint information (e.g. URI, FQDN, IP address) of the application server, which is part of the application functionalities. The purpose of the IE is the same in both specifications, and can be considered equivalent. |
| ACID(s) | O |  |  | The IE is not applicable in application registration in ETSI MEC. |
| EAS Provider Identifier | O | App Provider | M | 3GPP specification [2]: The identifier of the ASP that provides the EAS. ETSI MEC specification [14]: Provider of the application. It shall be consistent with the appProvider in the AppD, if an AppD is available. The purpose of the IE is the same in both specifications, and can be considered equivalent. |
| EAS Type | O | appCategory | O | 3GPP specification [2]: The category or type of EAS (e.g. V2X) ETSI MEC specification [14]: Category of the application. The purpose of the IE is the same in both specifications, and can be considered equivalent. |
| EAS description | O | AppD > appDescription | O | 3GPP specification [2]: Human-readable description of the EAS. ETSI MEC specification [14]: Human readable description of the MEC application. The purpose of the IE is the same in both specifications, and can be considered equivalent. |
| EAS Schedule | O |  |  | The IE is not applicable in application registration in ETSI MEC. |
| EAS Geographical Service Area | O |  |  | The IE is not applicable in application registration in ETSI MEC. |
| EAS Topological Service Area | O |  |  | The IE is not applicable in application registration in ETSI MEC. |
| EAS Service KPIs | O |  |  | The IE is not applicable in application registration in ETSI MEC. |
| EAS service permission level | O |  |  | The IE is not applicable in application registration in ETSI MEC. |
| EAS Feature(s) | O |  |  | The IE is not applicable in application registration in ETSI MEC. |
| EAS Service continuity support | O |  |  | The IE is not applicable in application registration in ETSI MEC. |
| List of EAS DNAI(s) | O |  |  | The IE is not applicable in application registration in ETSI MEC. |
| List of N6 Traffic Routing requirements | O |  |  | The IE is not applicable in application registration in ETSI MEC. |
| EAS Availability Reporting Period | O |  |  | The IE is not applicable in application registration in ETSI MEC. |
| EAS Status | O |  |  | The IE is not applicable in application registration in ETSI MEC. |
|  |  | appDId | O | ETSI MEC specification [14]: The application descriptor identifier. It is managed by the application provider to identify the application descriptor in a globally unique way. Shall be present if the application instance is instantiated by the MEC Management. |
|  |  | appInstanceId | O | ETSI MEC specification [14]: Identifier of the application instance. Shall be present if the application instance is instantiated by the MEC Management. |
|  |  | appServiceRequired | O | ETSI MEC specification [14]: Describes services a MEC application requires to run. ServiceDependency is defined in ETSI GS MEC 010-2 [13]. It shall shall not be provided if an AppD is available. |
|  |  | appServiceOptional | O | ETSI MEC specification [14]: Describes services a MEC application may use if available. ServiceDependency is defined in ETSI GS MEC 010-2 [13]. It shall shall not be provided if an AppD is available. |
|  |  | appFeatureRequired | O | ETSI MEC specification [14]: Describes features a MEC application requires to run. FeatureDependency is defined in ETSI GS MEC 010-2 [13]. It shall shall not be provided if an AppD is available. |
|  |  | appFeatureOptional | O | ETSI MEC specification [14]: Describes features a MEC application may use if available. FeatureDependency is defined in ETSI GS MEC 010-2 [13]. It shall shall not be provided if an AppD is available. |

NOTE 1:	The appInfo currently appears in the draft version of ETSI GS MEC 011 V3.0.6 (2022-03) [14]. In the current draft version, two additional IEs, isEAS and easProfile, are included in the appInfo to hold the place for providing EAS profile. These two IEs are not applicable to this solution, which analyses correspondence between IEs in EAS profile and appInfo.

This solution proposes to enable the application to perform registration on EES according to the mapping between appInfo and EAS profile.

NOTE 2:	The solution does not impact the procedures.

### 7.36.3	Solution evaluation

This clause provides an evaluation of the solution.

## 7.37	Solution #37: ACR request trigger timing

### 7.37.1	Architecture enhancements

None.

### 7.37.2	Solution description

This solution corresponds to the key issue #3 on enhancements to service continuity planning.

To support the determination of the ACR request trigger timing in case of service continuity planning, this solution proposes to introduce "General context holding time" as shown in table 7.37.2-2. This IE is an indication of the maximum time the EAS holds the application context for a UE to move to its service area after receiving an ACR notification from the EES following an ACR request from the EEC. "General context holding time" can be provided to the EES during the EAS registration request or EAS registration update (when there is a change).

Since the EASID of the EAS identifies the type of the application context (e.g. SA6Video, SA6Game etc) as described in TS 23.558 clause 7.2.4, "General context holding time” determined can depend on the EASID (type of the application context).

This IE can then be provided to the EEC during the discovery so that the EEC can use the time to decide when to make an ACR request. The EEC may provide "prediction expiration time" as in Solution#21 in the ACR request as per its predictions by making sure it does not exceed the "General context holding time".

"General context holding time" can be determined or updated due to factors such as resource usage and policy setting such as the policy of the ECSP providing the EHE where the EAS is hosted. In such a case, the T-EAS sends the EAS registration update request and the EES updates the "General context holding time" in the EAS profile.

Table 7.37.2-2: EAS Profile

| Information element | Status | Description |
| --- | --- | --- |
| EASID | M | The identifier of the EAS |
| EAS Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EAS. This information maybe discovered by EEC and exposed to ACs so that ACs can establish contact with the EAS. |
| General context holding time | O | The time that the EAS holds the context before the AC connects to the EAS in case of ACR for service continuity planning. It is an indication of the time the EAS holds the application context for a UE to move to its service area after receiving an ACR notification from the EES following an ACR request from the EEC. |
| ACID(s) | O | Identifies the AC(s) that can be served by the EAS |

#### 7.37.2.1	Procedure

#### 7.37.2.2	Enhancements to procedures in TS 23.558

The EEC can use the "General context holding time" to make a timely ACR request by performing an ACR launching procedure when it predicts to move to the service area of the discovered EAS within the time indicated in the IE. To reflect this, the procedures in service continuity scenarios in clause 8.8.2.2, clause 8.8.2.3, and clause 8.8.2.6 of 3GPP TS 23.558 [2] – Initiation by EEC using regular EAS Discovery, EEC executed ACR via S-EES, and EEC executed ACR via T-EES – are enhanced as follows:

#### 7.37.2.2.1	Enhancements to 'Initiation by EEC using regular EAS Discovery' in clause 8.8.2.2

4.	The EEC performs EAS discovery (as specified in clause 8.5) for the desired T-EASs by querying the T-EESs that were established in step 3 (or provided in the notification from the ECS – if it was the trigger). If EEC registration configuration for the EESs established in step 2 indicates that EEC registration is required, the EEC performs EEC registration with the EESs (as specified in clause 8.4.2.2.2) before sending the EAS discovery request. Step 5 is skipped if EAS discovery procedure results in only one discovered T-EAS.

When in step 1 the ACR for service continuity planning is triggered, and the "General context holding time" is included in the replied EAS discovery response, the EEC can make ACR request before it reaches respective T-EAS service area within the time period indicated by the IE.

5.	The AC and EEC select the T-EAS to be used for the application traffic.

NOTE 1:	Several EEC registrations with different EESs may result from T-EAS discovery process during a single ACR operation.

6.	The EEC performs ACR launching procedure (as described in clause 8.8.3.4) to the S-EES with the ACR action indicating ACR initiation and the corresponding ACR initiation data (without the need to notify the EAS). The S-EES may apply the AF traffic influence with the N6 routing information of the T-EAS in the 3GPP Core Network (if applicable), as described in clause 8.8.3.4. If the EEC has not subscribed to receive ACR information notifications for ACR complete events from the S-EES, the EEC subscribes for the notifications as described in clause 8.8.3.5.2.

#### 7.37.2.2.2	Enhancements to 'EEC executed ACR via S-EES' in clause 8.8.2.3

Phase I: ACR Detection

1.	The EEC detects that ACR may be required as described in clause 8.8.1. The EEC may detect that ACR may be required for an expected or predicted UE location in the future as described in clause 8.8.1.

Phase II: ACR Decision

2.	The EEC decides to proceed required procedures for triggering ACR.

Phase III:	ACR Execution

3.	The EEC determines the T-EES by using the provisioned information or performing service provisioning procedure per clause 8.3 of the present document. When in step 1 the ACR for service continuity planning is triggered, then the Connectivity information and UE Location in the Service Provisioning (as specified in clause 8.3) procedure contains the expected Connectivity information and expected UE Location. If the UE is within the service area of the T-EES, upon selecting T-EES the UE may need to establish a new PDU connection to the target EDN. If EEC registration configuration for the T-EES indicates that EEC registration is required, the EEC performs EEC registration with the selected T-EES as specified in clause 8.4.2.2.2. The EEC can then discover and select T-EAS by performing EAS Discovery with the T-EES per clause 8.5.2 of the present document.

When in step 1 the ACR for service continuity planning is triggered, and the "General context holding time" is included in the replied EAS discovery response, the EEC can make ACR request before it reaches respective T-EAS service area within the time period indicated by the IE.

NOTE 1:	Several EEC registrations with different EESs may result from T-EAS discovery process during a single ACR operation.

4.	The EEC performs ACR launching procedure (as described in clause 8.8.3.4) to the S-EES with the ACR action indicating ACR initiation and the corresponding ACR initiation data (with the need to notify the EAS). The S-EES authorises the request from the EEC. The S-EES decides to execute ACR based on the information received from the EEC, EEC context and/or EAS profile. The S-EES may apply the AF traffic influence with the N6 routing information of the T-EAS in the 3GPP Core Network (if applicable) and sends the ACR Notify message to the S-EAS to initiate ACT between the S-EAS and the T-EAS. If the EEC has not subscribed to receive ACR information notifications for ACR complete events from the S-EES, the EEC subscribes for the notifications as described in clause 8.8.3.5.2.

#### 7.37.2.2.3	Enhancements to 'EEC executed ACR via T-EES' in clause 8.8.2.6

Phase I: ACR Detection

1.	The EEC detects that ACR may be required as described in clause 8.8.1. The EEC may detect that ACR may be required for an expected or predicted UE location in the future as described in clause 8.8.1.

Phase II: ACR Decision

2.	The EEC decides to proceed with required procedures for ACR.

NOTE 1:	If supported, the AC can be involved in the decision. It is out of scope of the present document how the AC is involved.

Phase III:	ACR Execution

3.	The EEC determines the T-EES by using the provisioned information or performing service provisioning procedure per clause 8.3. When in step 1 the ACR for service continuity planning is triggered, then the Connectivity information and UE Location used in the service provisioning procedure contain the expected Connectivity information and expected UE Location. If the UE is within the service area of the T-EES, upon selecting the T-EES the UE may need to establish a new PDU connection to the target EDN. If EEC registration configuration for the T-EES indicates that EEC registration is required, the EEC performs registration with the selected T-EES as specified in clause 8.4.2.2.2. The EEC performs EAS Discovery with the T-EES per clause 8.5.2.

When in step 1 the ACR for service continuity planning is triggered, and the "General context holding time" is included in the replied EAS discovery response, the EEC can make ACR request before it reaches respective T-EAS service area within the time period indicated by the IE.

NOTE 2:	Several EEC registrations with different EESs may result from T-EAS discovery process during a single ACR operation.

4.	The EEC performs ACR launching procedure (as described in clause 8.8.3.4) to the T-EES with the ACR action indicating ACR initiation and the corresponding ACR initiation data (with the need to notify the EAS). If the received ACR initiation request contains an EEC context ID and the S-EES Endpoint, the T-EES performs an EEC Context Pull relocation (clause 8.9.2.2). The T-EES may apply the AF traffic influence with the N6 routing information of the T-EAS in the 3GPP Core Network (if applicable). Then the T-EES sends the ACR Notify message to the T-EAS. The EEC also subscribes to receive ACR information notifications for ACR complete events from the T-EES, as described in clause 8.8.3.5.2.

### 7.37.3	Solution evaluation

TBD.

## 7.38	Solution #38: ACR coordination

### 7.38.1	Architecture enhancements

None

### 7.38.2	Solution description

#### 7.38.2.1	General

The following solution corresponds to the key issue #19 on ACR scenario combination.

#### 7.38.2.2	Procedure

Multiple ACR scenarios can be selected and the detection entity can start detecting the need for ACR. In this solution, the decision-making entity (e.g. EEC, EES, EAS) of any selected scenario triggers the ACR. However, once the ACR procedure is triggered, the EES notifies the decision-making entities of other scenario(s) about the start of the ACR. Once the ACR procedure is completed, the EES notifies the decision-making entities of other scenario(s) about the completion of the ACR (success or failure). The decision-making entities of other scenario(s) may avoid triggering another ACR till one ACR procedure is in progress and notification about the completion of the ACR is received.

The solution proposes to enhance ACT status subscription procedure as specified in clause 8.8.3.6.2.3 of 3GPP TS 23.558 [2] and ACR information subscription as specified in clause 8.8.3.5.2 of 3GPP TS 23.558 [2] to subscribe for ACR execution start event.

Further, the solution proposes to enhance all scenarios specified in clause 8.8.2 of 3GPP TS 23.558 [2] to send notification of start of ACR execution phase as follows:

Pre-condition:

1)	More than one ACR scenarios are selected as specified in solution#19 or solution#35.

Figure 7.38.2.2-1 ACR coordination

1)	The ACR is triggered by one of the decision making entity (i.e. EAS or EES or EEC) and T-EAS is discovered.

2a)	If EEC or EES has initiated ACR, then the EES sends ACR management event notification (ACT Start) to the S-EAS indicating the start of the ACR execution. The notification message includes ACR identity (ACID, EEC ID (or UE ID), S-EAS endpoint and T-EAS endpoint).

2b)	If S-EAS or EES has initiated ACR, then the EES sends ACR information notification to EEC indicating the start of the ACR execution. The notification message includes ACR identity (ACID, EEC ID (or UE ID), S-EAS endpoint and T-EAS endpoint).

NOTE:	ACT initiation can be considered as a start of the execution phase.

3)	The ACR procedure is performed as specified in the scenario (in clause 8.8.2 of 3GPP TS 23.558 [2])

If concurrent ACR requests happens the EES rejects the additional ACR requests.

Enhancements to clause 8.8.3.5.3 ACR information notification procedure

Step 2 of the procedure will be modified as follows (in bold)

2.	The EES sends an ACR information notification to the EEC with the ACR information determined in step 1. The ACR information notification may include ACID to indicate the application context relocation of the AC is complete. If the S-EES has received the successful EEC Context Push response from T-EES, along with registration ID and the registration expiration time in the EEC Context Push relocation procedure, then the ACR information notification towards EEC also includes the registration ID and registration expiration time under EEC context relocation status (for successful status). Upon receiving the target information notification to indicate about start of the ACR execution, the EEC prevents triggering a second ACR execution for the same identity (ACID, EEC ID (or UE ID), S-EAS endpoint and T-EAS endpoint) until the current ACR execution is completed.

Enhancements to clause 8.6.3.2.3 Notify (for ACR management event notification)

Step 1-d will be modified as follows (bold):

d.	If "ACT start/stop" event is subscribed, during the ACR launch if the EEC indicates the need to notify the EAS in the ACR request as described in clause 8.8.3.4, the EES shall send notification to the EAS to inform it about the need to start or stop the ACT to or from another EAS. The notification message includes ACR identity (ACID, EEC ID (or UE ID), S-EAS endpoint and T-EAS endpoint).

2.	The EES sends ACR management event notification to the EAS. The EES includes the ACR management event notification information of the UE(s) and optionally the timestamp. If the event triggering the notification is DNAI change, the timestamp can be included to indicate the age of the user plane path management event notification information. The EES may only provide part of information included in the user plane path management event notification from 3GPP network (e.g. target DNAI). If the EAS had provided "Indication of EAS acknowledgement", the EES waits for acknowledgement from the EAS before it sends AF acknowledgement to the 3GPP core network.

If the event is "ACT start/stop", the notification shall include the endpoint address of the other EAS and the UE ID. Upon receiving the notification about the start of the ACR execution with "ACT start" event, The S-EAS prevents triggering a second ACR execution for the same identity (ACID, EEC ID (or UE ID), S-EAS endpoint and T-EAS endpoint) until the current ACR execution is completed.

### 7.38.3	Solution evaluation

The solution addresses Key issue #19. The solution proposes to coordinate among decision making entities to make sure single ACR remains in execution phase while allowing multiple entities to detect ACR for different scenarios. The solution proposes modify existing ACR scenarios to utilize existing event notifications to S-EAS or EEC at start of the execution phase. The solution also proposes to modify ACR information notification and ACR management event notification procedures. The solution is a viable solution.

# 8	Deployment scenarios

## 8.1	<Title>

Provide an illustrative description of the deployment scenario.

# 9	Involved entities and relationships

## 9.1	Federation and Roaming

### 9.1.1	General

This clause describes the relationship of edge computing service providers, PLMN operators, application service providers and end users, taking federation and roaming into account.

Figure 9.1.1: Relationships involved in edge computing service – federation and roaming

The end user is the consumer of the applications provided by the application service provider (ASP). End user:

-	can have ASP service agreement with a single or multiple application service providers.

-	has a PLMN subscription arrangement with a PLMN operator (HPLMN). The UE used by the end user can register on the HPLMN network and network of its roaming partners.

-	can have authorization to access edge services of a single or multiple ECSPs.

The ASP consumes the edge services (e.g., infrastructure, platform) provided by the ECSP. ASP:

-	can have edge computing service provider service agreement with a single or multiple ECSPs.

The PLMN operator provides connectivity between the end user and the edge services provided by the ECSP. PLMN operator:

-	can have the PLMN operator service agreement with a single or multiple ECSPs.

-	can have service agreement for roaming including agreements for Edge Computing services, and/or federation with a single or multiple PLMN operators.

The ECSP provides the edge services. ECSP:

-	can have PLMN operator service agreement with a single or multiple PLMN operators which provide edge computing support.

-	can have federation partnership to share edge services with a single or multiple ECSPs.

The ECSP and the PLMN operator can be part of the same organization.

# 10	Overall evaluation

This clause provides a summary of architecture enhancements and solution evaluations.

## 10.1	Architecture enhancements

### 10.1.1	Support for Terminal Equipment (TE)

The architecture enhancement captured as option#4 introduces support for TEs in the EdgeApp architecture. The interface between the TEs and the UEs running the EEC is out of scope of SA6.

With this enhancements ACs running on a TE will be able to communicate with the EEC running on a UE to avail services of the EEC over EDGE-5 interface. No specific enhancement over EDGE-5 is identified during the study to support this functionality.

## 10.2	Key issue evaluations

### 10.2.0	General

All the key issues and solutions specified in this technical report are listed in Table 10.2.0-1.

The table provides a mapping of the key issues to the related solutions. It also lists the dependencies on other working groups.

Table 10.2.0-1 Key issue and solutions

| Key issues (evaluation clause reference) | Solution | Solution (clause reference) |  |  |  | Dependency on other working groups |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Key issue #1: Enhanced notification service to the EEC | Solution #1: Service provisioning via push notification |  | 7.1 |  |  |  |  |
|  | Solution #3: Service provisioning triggering via SMS over NAS |  | 7.3 |  |  |  |  |
|  | Solution #20: Propagation of EEL notifications to EEC using Edge Notification Server |  | 7.20 |  |  |  |  |
| Key issue #2: Enablement of Service APIs exposed by EAS | Solution #8: EAS Service API enablement using CAPIF |  | 7.8 |  |  |  |  |
|  | Solution #11: A deployment option for alignment with ETSI MEC using CAPIF |  | 7.11 |  |  |  |  |
| Key issue #3: Enhancements to service continuity planning | Solution #6: ACR update in service continuity planning |  | 7.6 |  |  |  |  |
|  | Solution #7: EES monitors UE mobility for service continuity planning |  | 7.7 |  |  |  |  |
|  | Solution #12: Service continuity planning allowance |  | 7.12 |  |  |  |  |
|  | Solution #21: Prediction expiration time for service continuity planning enhancement |  | 7.21 |  |  |  |  |
| Key issue #4: EDGE-5 | Solution #22: Support simultaneous EAS connectivity in ACR |  | 7.22 |  |  |  | SA3 |
|  | Solution #34: EDGE-5 APIs |  | 7.34 |  |  |  |  |
| Key issue #5: Alignment of EDGEAPP and ETSI MEC | Solution #11: A deployment option for alignment with ETSI MEC using CAPIF |  | 7.11 |  |  |  | SA5 |
|  | Solution #36: Alignment of EDGEAPP and ETSI MEC |  | 7.36 |  | SA5 |  |  |
| Key issue #6: Edge services support across ECSPs | Solution #4: ECS discovery through serving ECS to support edge services across ECSPs |  | 7.4 |  |  |  |  |
|  | Solution #5: ECS enhancement to discover EESs via other ECSs to support edge services across ECSPs |  | 7.5 |  |  |  |  |
|  | Solution #13: Update ECS configuration information |  | 7.13 |  |  |  | SA2 |
| Key issue #7: Application traffic filter exposure | Solution #2: Traffic filter support for EDGE-3 API addressing application traffic detection |  | 7.2 |  |  |  |  |
| Key issue #8: EAS selection synchronization | Solution #15: Initial EAS selection declaration |  | 7.15 |  |  |  |  |
| Key issue #9: Enhancement of dynamic EAS instantiation triggering | Solution #32: Dynamic EAS instantiation triggering and notification |  | 7.32 |  |  |  | SA5 |
|  | Solution #33: Support for EEC Discovery of EAS(es) before instantiation |  | 7.33 |  |  |  | SA5 |
| Key issue #10: Support for roaming UEs | Solution #4: ECS discovery through serving ECS to support edge services across ECSPs |  | 7.4 |  |  |  | SA3 |
|  | Solution #5: ECS enhancement to discover EESs via other ECSs to support edge services across ECSPs |  | 7.5 |  |  |  | SA3 |
|  | Solution #13: Update ECS configuration information |  | 7.13 |  |  |  | SA2 |
|  | Solution #14: V-ECS Discovery via the H-ECS |  | 7.14 |  |  |  | SA3 |
| Key issue #11: ACR between EAS and Cloud Application Server | Solution #24: ACR between CAS and EAS |  | 7.24 |  |  |  |  |
|  | Solution #25: ACR between EAS and Cloud Application Server |  | 7.25 |  |  |  |  |
| Key issue #12: EEL service differentiation | Solution #12: Service continuity planning allowance |  | 7.12 |  |  |  |  |
|  | Solution #16: EAS discovery for different users |  | 7.16 |  |  |  |  |
| Key issue #13: Edge enabler layer support for EAS synchronization |  |  |  |  |  |  |  |
| Key issue #14: Application traffic influence for initially selected EAS | Solution #9: Application traffic influence trigger from EAS |  | 7.9 |  |  |  |  |
|  | Solution #15: Initial EAS selection declaration |  | 7.15 |  |  |  |  |
|  | Solution #17: Traffic influence for initial EAS discovery |  | 7.17 |  |  |  |  |
| Key issue #15: Support of constrained devices for Edge | Solution #10: low power mode support |  | 7.10 |  |  |  |  |
|  | Solution #18: Constraint device in EDGEAPP |  | 7.18 |  |  |  |  |
| Key issue #16: support of NAT deployed within the edge data network | Solution #23: UE identification with NAT |  | 7.23 |  |  |  | SA2, SA3 |
| Key issue #17: Discovery of a common EAS | Solution #27: Enabling AC Association Aware services by selecting common EASs |  | 7.27 |  |  |  |  |
|  | Solution #28: Common EAS discovery using EAS selection information |  | 7.28 |  |  |  |  |
|  | Solution #29: Discovery of a common EAS |  | 7.29 |  |  |  |  |
|  | Solution #30: Common EAS selection |  | 7.30 |  |  |  |  |
|  | Solution #31: Discover common EAS |  | 7.31 |  |  |  |  |
| Key issue #18: Linkage between EASs | Solution #26: Bundled EASs |  | 7.26 |  |  |  |  |
| Key issue #19: ACR scenario combination | Solution #19: EES determines the selected ACR scenario |  | 7.19 |  |  |  |  |
|  | Solution #35: EEC selected ACR scenarios |  | 7.35 |  |  |  |  |
| Key issue #20: Method of supporting federated EAS service |  |  |  |  |  |  |  |
| Key issue #21: Simultaneously EAS connectivity in ACR | Solution #22: Support simultaneous EAS connectivity in ACR |  | 7.22 |  |  |  |  |
| Key issue #22: EAS discovery in Edge Node sharing scenario |  |  |  |  |  |  |  |

### 10.2.1	Key issue #1: Enhanced notification service to the EEC

The open issues studied in the key issue #1 are as follows:

1.	Whether and how the EEC acquires the notification target address or a notification channel URI to receive the notifications?

2.	Whether and how EEC, ECS and EES support push notification mechanism. Whether and what additional functional entity is necessary for this?

3.	How are the EEC subscriptions and/or notification targets treated in mobility scenario (e.g. during ACR scenario)?

4.	Whether and how to utilize application triggering method specified in 3GPP TS 23.501 [5] to provide notifications to the EEC?

To address the open issues aforementioned, there are three solutions proposed as follows:

-	Solution #1: Service provisioning via push notification

-	It utilizes the push notification mechanism to enhance the service provisioning procedure, assuming that at least one push server is available to ECS and the UE has push function supporting the interaction with the push server.

-	Solution #3: Service provisioning triggering via SMS over NAS

-	It utilizes application triggering (device triggering) via SMS over NAS method specified in 3GPP TS 23.501 [5] to inform the EEC of the need to perform service provisioning.

-	Solution #20: Propagation of EEL notifications to EEC using Edge Notification Server

-	It proposes a centralized notification server, called Edge Notification Server, to enhance EEL's notifications delivery mechanism to EEC. The proposed Edge Notification Server interacts with EEC, EES and ECS respectively via the newly proposed interfaces as described in the clause 6.3.

The Solution #1 provides detailed operations of EEC and ECS to enable the service provisioning via the push notification mechanism.

The Solution #3 provides a detailed interaction between EEC and ECS to check if the SMS over NAS is supported and accompanied EDGE-8 operation of ECS for triggering EEC to perform service provisioning request.

The Solution #20 provides architecture enhancements using an Edge Notification Server, which is an optional functional entity. It also proposes a detailed role of the Edge Notification Server and procedures between ENS and other functional entities (EEC, EES and ECS). The Edge Notification Server functionality will be specified as generic notification service in SEAL TS 23.434 by generalizing to vertical applications. Edge Notification Server functionality leverages generic notification service from SEAL TS 23.434.

The above enhanced notification services utilizing push notification, application triggering via SMS over NAS and leveraging generic notification service from SEAL in lieu of Edge Notification Server will be addressed in the normative phase.

Push notification service in Solution #20 will be leveraged (including generic notification service from SEAL TS 23.434) by Solution #1, during service provisioning.

### 10.2.2	Key issue #2: Enablement of Service APIs exposed by EAS

The open issues studied in the key issue #2 are as follows:

1.	Identify any gaps in CAPIF to enable EAS Service APIs in the EDGEAPP architecture in terms of e.g., service-specific attributes for API publish/discovery, API availability subscription/notification across EES/CCF

2.	If any, whether and how to enhance CAPIF capabilities to address the gap identified as above

In order to address the open issues aforementioned, there are two solutions proposed as follows:

-	Solution #8: EAS Service API enablement using CAPIF

-	It exploits CAPIF as specified in 3GPP TS 23.222 [16] to support publication/discovery, and change subscription of EAS Service APIs for API invoking by the other EASs

-	Solution #11: A deployment option for alignment with ETSI MEC using CAPIF

-	It exploits CAPIF as specified in 3GPP TS 23.222 [16] as a deployment option for exposing EAS Service APIs to ETSI MEC entities (i.e., MEP and MEC applications)

The Solution #8 provides 1) a detailed role of EAS and EES as CAPIF entities in order to support publication/discovery, and change subscription of EAS Service APIs; and 2) a new IE, Service KPIs, for CAPIF APIs.

The Solution #11 provides a detailed role of EAS and EES as CAPIF entities in order to support discovery and invocation of EAS Service APIs for ETSI MEC entities.

EES supporting EAS's and other entity's (e.g., ETSI MEP or MEC App.) access to service APIs exposed by other EAS, in the context of CAPIF, will be addressed in the normative phase. The newly identified IE for CAPF APIs has an impact to 3GPP TS 23.222 [16] to be addressed in the normative phase.

### 10.2.3	Key issue #3: Enhancements to service continuity planning

The open issues studied in the Key Issue#3 are as follows:

Open Issues:

1.	How to rely on the capability of EES/EEC to detect whether the UE moves to the predicted location or not for service continuity planning?

2.	Whether and how the EEL can support the determination of the ACR request trigger timing in case of service continuity planning?

3.	How to deal with scenarios when the ACR needs to be modified, e.g. due to UE mobility?

4.	Whether and what additional capability exposure is required from the 5GS (e.g. NWDAF, OAM) to enhance the service continuity planning?

5.	Potential impact on information exchanged between EAS and EEL.

6.	Potential impact on information to communicate within the EEL.

This clause provides an overall evaluation of the key issue #3: Enhancements to service continuity planning. The solution #6, solution #7, solution#12, solution#21 cover different aspects for the open issue in the KI#3.

Solution#6 addresses the open issue #3 by introducing a new procedure to modify ACR parameters. Specifying the signaling required for the procedure are to be addressed in the normative phase (without impacting AC).

The solution #7 is used for solving the open issue#6. And solution #7 is applicable to the scenario where the EAS may not have the capability of monitoring the UE mobility for service continuity planning. Solution #7 will not impact the EDGEAPP architecture defined in the Rel 17.

The Solution #12 enables to allow the service continuity planning selectively for a given UE or an application. This principle of Solution # 12 is that the permission of service continuity planning can be determined by the EES for the purpose of service differentiation, then the EEC performs the required detection or acquisition of the planned or predicted UE mobility behaviour for the EASs allowed for service continuity planning according to the determined ACR modes. Further investigation in normative work is required for sol#12 to evaluate backward compatibility issue in clause 7.12.3.

### Solution #21 addresses KI#3 by including a prediction expiration time within ACR request from the EEC. This information can be used by T-EAS to adjust its waiting time for the UE to reach the service area. It is proposed to update the ACR request in normative phase to include prediction expiration time.

### 10.2.4	Key issue #4: EDGE-5

KI#4 required study of multiple open issues related to the interface between the Application Client and the Edge Enabler Client.

Solution #34 provides a set of procedures that fulfil the open issues. No need to modify the cardinalities of EDGE-5 interface were identified during the study. The solution provides methods for an AC to register to an EEC, perform EAS discovery, perform ACR related requests in different modes and subscribe for EEC capabilities. Architecture enhancement option#4 depicts how constrained devices such as TEs can utilize EDGE-5 to communicate with an EEC running on a UE.

No need was identified during the study to provide methods for discovery of EECs by the AC or for detection of abnormal termination of EEC or ACs.

Aspects related to mutual authentication, authorization and user consent are SA3's responsibility. Any recommendation from SA3 shall be considered during normative phase.

NOTE:	Coordination with CT groups is required for this KI.

### 10.2.5	Key issue #6: Edge services support across ECSPs

The open issues of this KI are as follows:

1.	Identify potential enhancements to the existing architecture defined in Rel-17 to enable inter-ECSP interactions.

2.	Study potential impact to support ECS discovery and service provisioning based on UE location.

3.	Whether and how EEC registers with an EES deployed by a partner ECSP?

4.	Study potential impact to support service continuity.

5.	How is EEC context continuity maintained across ECSPs with or without ACR?

6.	How the ECS can discover a T-EES having SLA with S-EES based on the federation agreements between ECSPs before EDGE-9 interaction?

To address the open issues the following solutions has been proposed:

-	Solution #4: ECS discovery through serving ECS to support edge services across ECSPs

  - -	After discovering another ECS2 which may have suitable EES, the ECS1 sends respond to the EEC or source EES with the ECS2 information. Then the EEC or source EES can send the request to the ECS2 directly to retrieve suitable EES.

  - -	Solution #4 relies on having sufficient information (e.g., ECSP policy, UE-specific service information, or ECSs information) configured or available in an ECS to determine candidate.

Editor's note:  Whether sufficient information (e.g., ECSP policy, or UE-specific service information, or ECSs information) can be configured in an ECS or not is FFS.

-	Solution #5: ECS enhancement to discover EESs via other ECSs to support edge services across ECSPs

  - -	The EEC or the EES gets the requested EES information from ECS2 via ECS1. If the ECS1 cannot discover a suitable EES to serve the UE at the current location (e.g., all the EESs registered on the ECS1 do not cover the given UE location), the ECS1 contacts another ECS2 which may have suitable EES and discovers the EES via ECS2.

  - -	Solution #5 relies on preconfigured information of ECS2 at ECS1.

Editor's note:  What is included in the preconfigured information and whether it is enough to determine candidate ECSs in step 2 is FFS.

-	Solution #13: Update ECS configuration information

  - -	This solution proposes to include additional optional IEs in the ECS configuration information. The information can be used in solutions 4 and 5 to find and select ECS2. ECS information shared in Solution 13 is limited to the ECSs whose information is available through H-PLMN.

Solution #5 is applicable to scenarios where the UE has home routing agreement with its service provider for service provisioning.

Solution #13 provides additional optional IEs to be used within solution #4 and #5 to find and select ECS2.

All the above solutions will use any agreed solution for determining ECS2 by ECS1.

Detailed signalling to provide ECS2 address to the requesting EEC or EES based on solution #4 is to be addressed in normative phase. Signalling between ECS1 and ECS2 to discover and retrieve provisioning to the requesting EEC or T-EES for the requesting EES will be addressed in normative phase according to solution #5. A new interface EDGE-10 between ECS1 and ECS2 is used in Solution #5. Also, in normative phase, if required, the additional optional IEs are added to ECS configuration information provided by ECS to 5GC according to Solution #13.

### 10.2.6	Key issue #7: Application traffic filter exposure

The open issues studied in the key issue #7 are as follows:

1.	How to support more application traffic filter for session with QoS API.

2.	How to support more application traffic filter for ACR management event API.

To address the open issues aforementioned, solution 2 is proposed as follows:

-	Solution #2: Traffic filter support for EDGE-3 API addressing application traffic detection

-	Allows the EAS to provide the domain name as traffic descriptor (indicates the applicable protocol and matching criteria) to the EES, which further invokes the PFD management procedure with the 3GPP CN as described in 3GPP TS 23.682 [10] and 3GPP TS 23.502 [8].

The Solution #2 provides detailed operation between EAS and EES for specific filters, and the EES invoking the PFD management procedure with the 3GPP CN.

### 10.2.7	Key issue #10: Support for Roaming UEs

This clause provides an overall evaluation for Key Issue #10, "Support for roaming UEs".

Solution #4, Solution #5, Solution #13, and Solution #14 all address aspects of Key Issue #10.

One aspect of Key Issue #10 is how the EEC in the roaming UE knows the availability of ECS(s) and discovers them in the VPLMN.

-	Solution #4 and Solution #14 share the principle that the home network can provide the EEC in the UE with information that is used to contact a V-ECS. In both solutions, this information is provided in the Service Provisioning Response.

-	In solution #4, the V-ECS information can be an address, endpoint or service API information.

-	In Solution #14, the V-ECS information can include an FQDN or an IP Address of a V-ECS.

-	In Solution #14, the V-ECS information can include a DNN (O) and/or S-NSSAI (O). The DNN / S-NSSAI can be used to establish an LBO PDU Session. Once the UE establishes an LBO PDU Session, Rel-17 procedures can be used to discover the V-ECS address.

-	In Solution #13, it is proposed that PLMN ID(s) can be sent with ECS Address Configuration Information. As in Rel-17, ECS Address Configuration Information is sent to the UE by the SMF during PDU Session Establishment and/or PDU Session Modification.

-	In Solution #5, V-ECS address information is not sent to the UE.

Another aspect of Key Issue #10 is how the EEC in the roaming UE knows the availability of EES(s) and discovers them in the VPLMN.

-	Solution #4, Solution #14, and Solution #13 share the principle that the EEC in the UE is provided with information to contact ECS(s) in the VPLMN. The EEC can then contact an ECS in the VPLMN to discover EES(s) in the VPLMN.

-	In Solution #5, the H-ECS uses the EDGE-10 interface to obtain information from the V-ECS about what EES(s) are available in the VPLMN. The H-ECS then sends the information about the EES(s) in the VPLMN to the EEC in the Service Provisioning Response.

Another aspect, which is addressed by all Key Issue #10 solutions, is what information the HPLMN uses to determine what information about edge computing services in the VPLMN to send to the UE.

-	In Solution #13, ECS Address Configuration Information(s) can be stored in the UE's subscription and PLMN ID(s) can be stored with each ECS Address Configuration Information. The SMF sends all ECS Address Configuration Information(s) and PLMN ID(s) from the UE's subscription to the UE. The UE can determine what ECS Address Configuration Information(s) to use based on what PLMN it is registered to.

-	Solution #4, Solution #5, and Solution #14 share the principle that the ECS uses the PLMN ID of the PLMN where the UE is currently registered to determine what information about edge computing services in the VPLMN to send to the UE.

-	The Rel-17 Service Provisioning Request can already include the UE Location. The UE Location can be expressed as a Tracking Area Identity. The Tracking Area Identity includes a PLMN ID.

-	Solution #4 and Solution #5 share the principle that, if location is not included in the Service Provisioning Request, the ECS interacts with the Core Network to obtain the UE's location. Allowing the ECS to obtain the UE's location in this manner can be done with minimal impact to normative specifications. For example, the ECS can obtain the UE's location by invoking existing NEF API(s).

-	In Solution #4, Solution #5 and Solution #14, if a VPLMN ID is not included in the Service Provisioning Request, the ECS obtains the PLMN ID by interacting with the Core Network. Solution #14 explains that the ECS can do this by invoking the NEF's monitoring event API with the monitoring type set to ROAMING_STATUS and the plmnIndication set to TRUE as described in 3GPP TS 29.522 [17] and 3GPP TS 29.122 [18]. Allowing the ECS to obtain the UE's PLMN ID in this manner can be done with minimal impact to normative specifications. For example, the ECS can obtain the UE's location by invoking the existing NEF monitoring event API.

- 	Solution #4 relies on having sufficient information (e.g., ECSP policy, or UE-specific service information, or ECSs information) configured or available in an ECS to determine candidate ECSs.

Editor's note:	Whether sufficient information (e.g., ECSP policy, or UE-specific service information, or ECSs information) can be configured in an ECS or not is FFS.

- 	Solution #5 relies on preconfigured information of ECS2 at ECS1.

Editor's note:	What is included in the preconfigured information and whether it is enough to determine candidate ECSs in step 2 is FFS.

NOTE:	It cannot be assumed that all ECS(s) will have access to NEF API(s).

Another aspect, which is addressed by all Key Issue #10 solutions, is how the HPLMN obtains the information about edge computing services in the VPLMN to send to the UE. Solution #4, Solution #5, Solution #13, and Solution #14 all require that the home network (i.e. the H-ECS or H-SMF) determine what V-ECS(s) that are used by the EEC. In Solution #4, Solution #13, and Solution #14 the information about the V-ECS(s) is sent to the EEC. In Solution #5, the V-ECS(s) are contacted by the H-ECS to obtain EES information to the send to the EEC. In Solution #4, Solution #5, Solution #13, and Solution #14, the home network (i.e. the H-ECS or H-SMF) must determine what V-ECS(s) that are used by the UE.

-	In Solution #13, ECS Address Configuration Information(s) is stored in the UE's subscription as is done in Rel-17. The information for each V-ECS that the UE can use would need to be stored in the UE's subscription. ECS information shared in Solution 13 is limited to the ECSs whose information is available through H-PLMN.

-	In Solution #4, the ECS of the HPLMN uses the location information and/or VPLMN ID to discover what V-ECS information (e.g. address, endpoint or service API information) to send to the UE. The discovery can be based on pre-configuration or the H-ECS can discover service API information exposed by that ECS via CAPIF discovery procedure as specified in TS 23.222 [16]

-	In Solution #5, the ECS of the HPLMN uses the location information and/or VPLMN ID to discover V-ECS information (e.g. address, endpoint or service API information). The discovery can be based on pre-configuration or the H-ECS can discover service API information exposed by that ECS via CAPIF discovery procedure as specified in TS 23.222 [16]. The H-ECS then queries the V-ECS to obtain EES information to send to the UE. It is assumed that the ECS uses existing Rel-17 mechanisms from TS 23.558 [2] to determine what EES information to send to the EEC. For example, the ECS can identify the EES(s) based on the provided AC profile(s), the UE location, UE-specific service information, or an ECSP policy.

-	In Solution #14, the ECS of the HPLMN uses the VPLMN ID to discover V-ECS information (e.g. address, endpoint or service API information). The H-ECS then uses the EDGE-10 interface to query the V-ECS to obtain information to send to the UE. The query can include the UE's location and the information that is sent to the UE can be a DNN / S-NSSAI combination that can be used by the UE to establish an LBO PDU Session in the VPLMN. The V-ECS can use the UE's location to determine what DNN / S-NSSAI combination that is sent to the UE in order to cause the UE to establish an LBO PDU Session. The H-ECS can use the PLMN ID to determine the V-ECS to contact. For example, this can be based on a DNS lookup or the addresses of V-ECSs can have been pre-configured in the H-ECS (e.g. via OAM). If the VPLMN hosts multiple V-ECSs, then which V-ECS is resolved can be based on configuration.

-	In solution #14, how H-ECS uses the IEs in service provisioning request, other than UE location, in its request towards V-ECS is not specified. Also, how V-ECS selects the ECS in V-PLMN that will serve the EEC is not specified.

Editor's note: Whether handling of IEs in service provisioning request such as AC profiles, and selection of ECS(s) by V-ECS needs to be specified is FFS.

-	Solution #5 and Solution #14 both involve interaction between the V-ECS and H-ECS and both solutions largely rely on pre-configuration for how the H-ECS determines what V-ECS to contact. Since there can be multiple ECSs (e.g., from different ECSPs) deployed in the V-PLMN, it can be crucial to contact appropriate ECS based on EEC's requirements provided in the service provisioning request or EES's requirements provided in the retrieve T-EES request, e.g., in case of service continuity situation. Also, EDGE-10 enhancements based on solutions to other key issues (e.g. Key Issue #6) might be considered in order to allow the H-ECS to perform a more dynamic discovery, or determination, of the V-ECS to contact.

Editor's note: This evaluation does not discuss filtering and/or selection of the ECS information available in the V-PLMN. If needed, this evaluation will be updated to include discussion of this issue.

An important point of Key issue #10 is that the UE needs to obtain information for V-ECSs (ECS available in VPLMN) to obtain service provisioning information in VPLMN.

-	In solution #13, the new information that is sent to the UE is the PLMN ID(s) with ECS Address Configuration Information.

-	In solution #4, the new information that is sent to the EEC in the UE is V-ECS address, endpoint or service API information.

-	In solution #5, the new information that is sent to the EEC in the UE is VPLMN EES Information in the Service Provisioning Response from the H-ECS.

-	In solution #14, the new information that is sent to the EEC in the UE is a V-ECS address or a DNN / S-NSSAI combination that can be used by the EEC to establish an LBO Session in the VPLMN.

When the EEC contacts an H-ECS, there are cases where the H-ECS can detect that the UE is roaming and can obtain service provisioning from a V-ECS. In Rel-18, the Service Provisioning Response will be updated to allow the H-ECS to provide a V-ECS information to the EEC on EDGE-4. As described in Solutions #4 and #14, the V-ECS information sent to the EEC can include address (FQDN or an IP Address), endpoint or service API information of a V-ECS.

There are deployment scenarios where the EEC needs to access the ECS via an LBO session. There are cases where the UE is not pre-configured with an DNN / S-NSSAI combination that can be used to establish an LBO PDU session that can be used to reach the V-ECS. Thus, as described in Solution #14, the Service Provisioning Response will be updated to allow the H-ECS to provide a DNN / S-NSSAI combination to the EEC. The DNN / S-NSSAI combination can be used by the EEC to establish an LBO PDU Session in the VPLMN. Per existing Rel-17 behaviour and as defined in TS 23.548 [19], if the EEC does not have contact information for a V-ECS, the EEC can obtain V-ECS Address Configuration information from the V-SMF during PDU Session Establishment.

In Rel-17, it is already possible for an H-SMF to provide ECS Address Configuration information to the UE during PDU Session Establishment or in a PDU Session Modification Command. As explained in TS 23.548 [19] "The UDM in the HPLMN can provide the SMF (in HPLMN in HR case, in VPLMN in LBO case) with ECS address configuration information that depends on the serving PLMN of the UE." The Rel-17 approach requires that the UDM detect when the UE's serving PLMN changes and only sends the UE ECS Address Configuration information that is associated with the PLMN that is currently serving the UE. In Rel-18, the principles of Solution #13 will be followed so that the UE ECS Address Configuration information that is sent to the UE during PDU Session Establishment and PDU Session Modification can include PLMN ID(s).

As described in Solution #5, there are cases where the H-ECS is able to provision the EEC with information about EES(s) that are in the VPLMN. The H-ECS can use the EDGE-10 to obtain information from the V-ECS about what EES(s) are available in the VPLMN. The principles of Solution #5 will be followed to enhance the Service provisioning procedure to allow the H-ECS to provide the EEC with information about EES(s) in the VPLMN.

### 10.2.8	Key issue #11: ACR between EAS and Cloud Application Server

There are two solutions in the TR, CES-less solution is described in solution #25, and solution with CES is described in solution #24.

With CES, ACR feature parity can be supported in the ACR btw. CAS and EAS utilizing all EDGEAPP developed features (e.g. AS discovery, AS registration). Comparing to CES-less solution, it needs a new function entity in the central DN to support CAS, and CES is part of the EEL. The CES has the same functions as the EES without having service area restriction. The CAS registers in CES in order to be discoverable by the EEC using EDGEAPP EAS discovery mechanism, the CES registers in ECS in order to be discoverable by the EEC using EDGEAPP service provisioning procedure. CAS registration to CES is an additional functionality that need to be supported for all CAS(s).

For CES-less solution as described in solution #25, it has a mixed use of regular DNS query and EDGEAPP EAS discovery. When a T-EAS cannot be discovered using the EDGEAPP mechanism, EDGEAPP entities (e.g. EEC) falls back to regular DNS query. It supports ACR scenarios as described in solution #25 for ACR from EAS to CAS, it also supports ACR scenarios for ACR from CAS to EAS.

Both solutions can be considered in the normative work, the CES can be considered an optional entity in the EDGEAPP architecture. They share the same EDGEAPP EAS discovery/service provisioning procedure when trying to discover appropriate EAS. If there is CES available and registered in the ECS in the network, the EDGEAPP mechanism returns CES in service provisioning response to the EEC/S-EES or the S-EAS obtains CAS from CES via S-EES and EDGEAPP mechanism follows; otherwise, regular DNS is used to find CAS due to EDGEAPP mechanism failure (no EES configuration or T-EAS is not discoverable).

Editor's note:	Further evaluation is FFS according to the resolution of the ENs in clause 6.5 and 6.6.

### 10.2.9	Key issue #14: Application traffic influence for initially selected EAS

This clause provides an overall evaluation of the key issue #14 Application traffic influence for initially selected EAS. The solution #9, solution #15 and solution #17 are complementary to each other.

The solution #9 is applicable to the scenario where the EAS triggers the EES to perform traffic influence without necessitating requests to be made on a per UE basis.

The solution #15 is applicable to the scenario where the EEC (or AC and EEC) selects the EAS after EAS discovery and then provides a selected EAS declaration request to the EES, which may be used by the EES to make traffic influence decisions if those haven't already been performed.

The solution #17 is applicable to the scenario where the EES selects the EAS and performs traffic influence immediately for the selected EAS after EEC sent EAS discovery request based on attributes in the EEC provided AC profile, providing the selected EAS to the EEC in the EAS discovery response.

All 3 solutions result in network resources being reserved to varying degrees before actual use, where this may be lower in the UE specific solutions of solution 15 and 17.

Editor's note:	Further evaluation is FFS according to the resolution of the ENs in clause 7.9 and clause 7.15.

### 10.2.10	Key issue #18: Linkage between EASs

The open issues of key issue #18 are as follows:

-	How can the EEL identify linkages between the EASs?

-	What are the impacts on EEL procedures due to linkage between the EASs e.g. when the linked EASs are served by the same EES and require ACR due to UE mobility?

Solution #26 solves these open issues by enhancing the AC, EAS and EES profiles. Enhancements include adding EAS bundle information and EAS bundle requirements in the identified profiles. The solution also enhances EAS discovery filters and Retrieve EES request. Further, handling of these IEs at the EEC, EES and ECS is also described.

### 10.2.11	Key issue #21: Simultaneously EAS connectivity in ACR

Solution #22 enhances AC profile and EDGE-1 ACR procedure to enable EES to influence the traffic for simultaneous EAS connectivity during service continuity. The detailed impact to 3GPP TS 23.558 [2] and remaining EN in clause 7.22.2.1 for SA2 coordination will be addressed in the normative phase.

### 10.2.x	Key issue #x

This clause provides an overall evaluation of all the solutions defined for Key Issue #x.

# 11	Conclusions

## 11.1	General

This technical report fulfils the objectives of the study on application architecture for enabling Edge Applications. The report includes the following:

1.	Definition of terms and abbreviations used in the study (clause 3);

2.	Key issues identified by the study (clause 4) and corresponding architectural requirements (clause 5);

3.	Enhancements to edge application architecture specified in 3GPP TS 23.558, corresponding to the key issues and architectural requirements (clause 6);

4.	Individual solutions addressing the key issues (clause 7);

5.	Set of deployment options (clause 8) and updated business relationships considering Federation and Roaming support (clause 9); and

7.	Overall evaluations of all the solutions (clause 10).

Some of the individual solutions have dependency on other working groups within 3GPP. This dependency is summarized in overall evaluations (clause 10).

## 11.2	Conclusions for normative work

### 11.2.1	General conclusions

The study concludes with following general considerations for the normative work:

1.	Definition of terms and abbreviations captured in clause 3 will be reused;

2.	Architectural requirements identified in clause 5 will be used for updated baseline architectural requirements; and

3.	Deployment scenarios and involved business relationships will be considered as captured in clause 8 and clause 9 respectively. Additional deployment models and their implications on the solutions will be considered.

### 11.2.2	Architecture enhancement conclusions

The study concludes with following architectural enhancements considerations for the normative work:

1.	Architecture enhancements from clause 6 corresponding to the concluded solutions will be used for updating baseline edge application architecture specified in 3GPP TS 23.558:

a.	architecture corresponding to solution #20 as specified in clause 6.3 and additionally leveraging generic notification service from SEAL.

b.	architecture enhancement as specified in clause 6.4

### 11.2.3	Solution conclusions

The study concludes with following solution considerations for the normative work:

1.	Following individual solutions, corresponding to the key issues, will be considered as candidate solutions:

i.	for Key issue #1 (Enhanced notification service to the EEC):

a.	Solution #1: Service provisioning via push notification

-	SEAL Notification Service will be specified in TS 23.434 to support the Solution #1

-	The usage of SEAL Notification Service in TS 23.558 will be captured to Solution #1

b.	Solution #3: Service provisioning triggering via SMS over NAS

-	The usage of SMS over NAS to trigger service provisioning procedure will be specified in TS 23.558 as per Solution #3.

c.	Solution #20: Propagation of EEL notifications to EEC using Edge Notification Server

-	SEAL Notification Service will be specified in TS 23.434 to support the Solution #20

-	The usage of SEAL Notification Service in TS 23.558 will be captured to Solution #20

ii.	for Key issue #2 (Enablement of Service APIs exposed by EAS):

a.	Solution #8: EAS Service API enablement using CAPIF

iii.	for Key issue #3 (Enhancements to service continuity planning):

a.	Solution #x (<<title>>)

iv.	for Key issue #4 (EDGE-5):

a.	Solution #34 (EDGE-5 APIs)

v.	for Key issue #5 (Alignment of EDGEAPP and ETSI MEC):

a.	Solution #x (<<title>>)

vi.	for Key issue #6 (Edge services support across ECSPs):

a.	Solution #x (<<title>>)

vii.	for Key issue #7 (Application traffic filter exposure):

a.	Solution #2: Traffic filter support for EDGE-3 API addressing application traffic detection

viii.	for Key issue #8 (EAS selection synchronization):

a.	Solution #x (<<title>>)

ix.	for Key issue #9 (Enhancement of dynamic EAS instantiation triggering):

a.	Solution #x (<<title>>)

x.	for Key issue #10 (Support for roaming UEs):

a.	Solution #x (<<title>>)

xi.	for Key issue #11 (ACR between EAS and Cloud Application Server):

a.	Solution #x (<<title>>)

xii.	for Key issue #12 (EEL service differentiation):

a.	Solution #x (<<title>>)

xiii.	for Key issue #13 (Edge enabler layer support for EAS synchronization):

a.	Solution #x (<<title>>)

xiv.	for Key issue #14 (Application traffic influence for initially selected EAS):

a.	Solution #x (<<title>>)

xv.	for Key issue #15 (Support of constrained devices for Edge):

a.	Solution #x (<<title>>)

xvi.	for Key issue #16 (Support of NAT deployed within the edge data network):

a.	Solution #x (<<title>>)

xvii.	for Key issue #17 (Discovery of a common EAS):

a.	Solution #x (<<title>>)

xviii.	for Key issue #18 (Linkage between EASs):

a.	Solution #26 (Bundled EASs)

xix.	for Key issue #19 (ACR scenario combination):

a.	Solution #x (<<title>>)

xx.	for Key issue #20 (Method of supporting federated EAS service):

a.	Solution #x (<<title>>)

xxi.	for Key issue #21 (Simultaneously EAS connectivity in ACR):

a.	Solution #x (<<title>>)

xxii.	for Key issue #22 (EAS discovery in Edge Node sharing scenario):

a.	Solution #x (<<title>>)

2.	Individual solutions, not listed under bullet 1 may be adopted in technical specification with appropriate enhancements.

Annex A (Informative):
ETSI MEC and EDGEAPP system comparison

## A.1	General

This Annex compares EDGEAPP architecture R17 as defined in 3GPP TS 23.558 [2] and ETSI MEC architecture [3] and provides a gap analysis, addressing the objective in Key issue #5 in clause 4.5.

## A.2	Service consumer and service provider

The functionalities enabled via the Mp1 reference point between MEC applications and MEC platform is mainly described in ETSI GS MEC011 [14]. The related functionality includes MEC service registration/deregistration, MEC service discovery and event notifications. Other functionality includes MEC service availability, traffic rules, DNS and time of day.

From ETSI MEC's perspective, there are two types of MEC Applications, i.e., MEC Application that consumes MEC Services and MEC Application that provides MEC service(s). For the MEC Application that provides MEC service(s), the MEC Application sends a service registration request to the MEC platform to register the MEC service during the MEC Application start-up procedure. As for the MEC Application that consumes MEC Services, the MEC Application can send a service query request to the MEC platform to discover a MEC service. It should be noted that the API of registration and discovery is defined for MEC service.

In R17 of SA6, the EES can take the role of the CAPIF core function, and the vertical application enabler server acting the AEF and publish the vertical application enabler server APIs to the EES. Further, the vertical application enabler server APIs is discovered by the EASs acting as the API invoker during the service API discover procedure as specified in 3GPP TS 23.222 [16].

In clause 4.2, the Key issue #2 plans to study Enablement of Service APIs exposed by EAS. The R17 of EDGEAPP only defines the functionality of EAS acting as an invoker, which is similar to MEC Application that consumes MEC Services defined in ETSI MEC:

[Observation A.2-1] The R17 of EDGEAPP only defines the functionality of EAS acting as an invoker, which is similar to MEC Application that consumes MEC Services defined in ETSI MEC. According to the Key issue #2 in clause 4.2, The EAS acting as a service provider is expected to be defined in R18 and expose service APIs.

[Observation A.2-2] According to the Key issue #2 in clause 4.2, the EAS can act as a service provider and EES can act as CAPIF core function so different services will be discoverable at different EESs. How the information of a service registered at one MEC platform is made available to other platforms in the same MEC system is not explicitly specified within ETSI MEC, while in EDGEAPP, as EES supports CAPIF core function, the EAS service published on EES1 can be discovered by EAS registered on EES2 through CAPIF-6 or CAPIF-6e.

## A.3	EAS/MEC application profile provisioning

ETSI MEC and EDGEAPP defined different style of EAS/MEC application profile provisioning. The information flows for lifecycle management of MEC applications is described in ETSI GS MEC010-2 [13]. The informational flows for the optional MEC Application registration are described in ETSI GS MEC 011 [14]. The MEC application can start producing or consuming MEC Services after the MEC Application is instantiated and running. The application information (AppInfo), which can be regarded as the MEC application profile, represents the information provided by the MEC application instance as part of the "application registration request" message. The attributes of the AppInfo are available from the clause 7.1.2.6 of ETSI GS MEC011 [14]:

Some fields in AppInfo are intentionally not duplicating the EAS profile (if present) with conflicting parameters but should be consistent with them. This is highlighted in NOTE 1 and NOTE 2, for example. It can be seen that unlike AppD [13], which is mainly used in the management plane for instantiating an application, and is static in nature, AppInfo carries the runtime information about the MEC application instance.

In EDGEAPP, the EAS profile is provided in the EAS registration request. According to clause 8.2.4 of 3GPP TS 23.558 [2] the information element of the EAS profile is listed as below:

Table A.3-2: EAS Profile

| Information element | Status | Description |
| --- | --- | --- |
| EASID | M | The identifier of the EAS |
| EAS Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EAS. This information maybe discovered by EEC and exposed to ACs so that ACs can establish contact with the EAS. |
| ACID(s) | O | Identifies the AC(s) that can be served by the EAS |
| EAS Provider Identifier | O | The identifier of the ASP that provides the EAS. |
| EAS Type | O | The category or type of EAS (e.g. V2X) |
| EAS description | O | Human-readable description of the EAS |
| EAS Schedule | O | The availability schedule of the EAS (e.g. time windows) |
| EAS Geographical Service Area | O | The geographical service area that the EAS serves. ACs in UEs that are located outside that area shall not be served. |
| EAS Topological Service Area | O | The EAS serves UEs that are connected to the Core Network from one of the cells included in this service area. ACs in UEs that are located outside this area shall not be served. See possible formats in Table 8.2.7-1. |
| EAS Service KPIs | O | Service characteristics provided by EAS, detailed in Table 8.2.5-1 |
| EAS service permission level | O | Level of service permissions e.g. trial, gold-class supported by the EAS |
| EAS Feature(s) | O | Service features e.g. single vs. multi-player gaming service supported by the EAS |
| EAS Service continuity support | O | Indicates if the EAS supports service continuity or not. This IE also indicates which ACR scenarios are supported by the EAS. |
| List of EAS DNAI(s) | O | DNAI(s) associated with the EAS. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [14]. It is a subset of the DNAI(s) associated with the EDN where the EAS resides. |
| List of N6 Traffic Routing requirements | O | The N6 traffic routing information and/or routing profile ID corresponding to each EAS DNAI. |
| EAS Availability Reporting Period | O | The availability reporting period (i.e. heartbeat period) that indicates to the EES how often it needs to check the EAS's availability after a successful registration. |
| EAS Required Service APIs | O | A list of the Service APIs that are required by the EAS |
| EAS Status | O | The status of the EAS (e.g. enabled, disabled, etc.) |

Comparison of AppInfo in 7.1.2.6 of ETSI GS MEC 011 [14] and EAS Profile in Table A.3.2 shows that:

[Observation A.3-1] Both ETSI MEC and EDGEAPP provides similar types of EAS/MEC application profile provisioning. Both the EAS Profile and MEC Application Instance (AppInfo) is provided during the application registration request.

[Observation A.3-2] Some IEs of the EAS profile may overlap with the ones defined in AppInfo (e.g., EAS ID vs. appName, EAS Provider Identifier vs. appProvider, EAS Endpoint vs. endpoint). Both AppInfo and EAS Profile has many optional IEs, whether and how to address their differences in SA6 is FFS.

## A.4	EAS registration and EAS discovery

In R17 of EDGEAPP, the EAS Registration procedure is defined to allow an EAS to provide its information to an EES in order to enable its discovery as defined in clause 8.4.3 of 3GPP TS 23.558 [2]. The EAS discovery procedure is used to provide EAS information to the EEC. After the EEC is provisioned with the EAS information, it can establish a connection to the EAS. Besides, in the service continuity scenario, the source EAS may send an EAS discovery request to the EES to discover a target EAS (providing same functionality as the source EAS) to serve the UE as defined in clause 8.8.3.2 of 3GPP TS 23.558 [2].

However, in current ETSI MEC specification, no APIs for MEC Application registration is defined because it is assumed that all MEC Application are on-boarded and managed by MEC Orchestrator, which was specified in ETSI GS MEC 010-2 [13]. API for MEC Application discovery is not defined since the existing MEC service is either defined from the MEC Application's perspective or it is consumed by the MEC Application rather than the UE.

Therefore, the comparison EAS registration and EAS discovery of EDGEAPP [2] and ETSI MEC specification [13] shows that:

[Observation A.4-1] The EAS registration and EAS discovery mechanism is defined in R17 of SA6 and ETSI MEC introduced MEC application registration (ETSI GS MEC 011 v3.0.6). It is FFS whether and how to address such differences in SA6, e.g., in support of ETSI MEC.

[Observation A.4-2] ETSI MEC platform(MEP) supports service registration. In the registration parameter "ServiceInfo"，there is a mandatory field "consumedLocalOnly" used to indicate that the service can only be consumed by the MEC applications located in the same locality, which means ETSI MEC services (produced by Authorized MEC APPs) registered and exposed on MEP can be invoked by MEC consumer APPs deployed on the same or another MEC host.

Annex B (Informative): Deployment and Evolution options of EDGEAPP and ETSI MEC platforms

## B.1	General

This clause provides the analysis to address the first open issue of KI#5, which intends to study and analyse different deployment options of EDGEAPP and ETSI MEC platforms. In that regard, this clause describes the following foreseen types of deployment and evolution scenarios for deployment of EDGEAPP and ETSI MEC.

Annex C of TS 23.558 provides a relationship between EDGEAPP and ETSI MEC architectures as in figure below.

Figure B.1-1: Relationship in EDGEAPP and ETSI MEC architecture

Editor's note: Whether and how to enhance EDGE-9 or Mp3 is FFS.

## B.2	Deployment options

### B.2.1	Deployment Option-1: Collocated Platforms

Based on Figure B.1-1, the two platforms (EES and MEC Platform) are co-located, and made by a single (unique) equipment, which is compliant with both standards. However, the actual deployment details of two platforms is implementation specific (Figure B.2.1-1).

Figure B.2.1-1: EES and MEC Platform as two different AFs on a single Physical/NFV Infrastructure

### B.2.2	Deployment Option-2: Converged architecture

#### B.2.2.1	General

From the practical and business perspective, it is possible that an operator has deployed ETSI MEC architecture in its MEC sites to provide edge service since the stage 1 work of ETSI MEC has been already finished for a period of time. At the same time, the operator still cannot deploy EDGEAPP architecture since the stage 3 work of EDGEAPP is still not completed at the time being.

On the other hand, it is assumed that an enhanced architecture, including a converged architecture as depicted in Figure B.2.2-1, will be introduced after completion of release 18. The converged architecture is expected to satisfy the following requirement:

-	The MEP+EES is able to satisfy all the functionalities of MEP defined in ETSI and EES defined in SA6.

-	A uniform API is defined for the EAS and MEC app, i.e., EDGE-3 and Mp1 are unified into one interface and the EAS and MEC app will consume the same service from the MEP+EES.

-	EDGE-9 and Mp3 are unified into one interface.

NOTE: Management of MEP+EES is under the scope of SA5.

Figure B.2.2.1-1 Converged architecture for EDGEAPP and ETSI MEC alignment

The two platforms (EES and MEC Platform) are co-located which can be implemented as a single AF (e.g. realized as one VNF) compliant with both standards (Figure B.2.2-2).

Figure B.2.2.1-2: EES and MEC Platform as a single AF on a single NFV Infrastructure (NFVI)

#### B.2.2.2	Evolution Options

##### B.2.2.2.1	General

In this clause the converged architecture is the architecture described in clause B.2.2 and satisfies the requirements listed in that clause.

##### B.2.2.2.2	Evolution Option #1- Enhancement of a deployed MEP to support the functionality of EES

After the completion of Release 18, an operator may determine to upgrade/evolve the ETSI MEC architecture to the converged architecture if the operator deployed ETSI MEC architecture in its MEC sites to provide edge service in the early stage. The MEP needs to be upgraded to the MEP+EES for supporting the functionality of EES defined in 3GPP SA6. The deployment and evolution scenario is depicted in Figure B.2.2.2.2-1.

Figure B.2.2.2.2-1 Evolution Option #1- An early stage deployed MEP is enhanced to support the functionality of EES in a later stage.

In Figure B.2.2.2.2-1, the EDN_1 and EDN_2 are deployed by the same operator.

##### B.2.2.2.3	Evolution Option #2 Enhancement of a deployed EES to support the functionality of MEP

After the completion of Release 18, an operator may opt to upgrade/evolve the EDGEAPP architecture to the converged architecture if the operator deployed EDGEAPP architecture in its EDNs to provide edge service in the early stage. The deployment and evolution scenario is depicted in Figure B.2.2.2.3-1.

Figure B.2.2.2.3-1  Evolution Option #2- Enhancement of a deployed EES to support the functionality of MEP

### B.2.3	Deployment Option-3: non-Collocated Platforms

The two platforms are non-collocated, and reside in two different data networks, where EES is in the Mobile Network Operator (MNO) domain while the ETSI MEC platform is in another MNO domain (Figure B.2.3-1).

Figure B.2.3-1: EES and MEC Platform as two different AFs in two different EDNs

Annex C (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-07 | SA6#44-e | S6-211758 |  |  |  | TR Skeleton | 0.0.0 |
| 2021-07 | SA6#44-e |  |  |  |  | S6-211754, S6-211759, S6-211760 | 0.1.0 |
| 2021-09 | SA6#45-e |  |  |  |  | S6-212048, S6-212054, S6-212087, S6-212103, S6-212115, S6-212142, S6-212166, S6-212167, S6-212168, S6-212169, S6-212170, S6-212171 | 0.2.0 |
| 2021-10 | SA6#45-BIS-e |  |  |  |  | S6-212478, S6-212402, S6-212479, S6-212480, S6-212481, S6-212482, S6-212207, S6-212411, S6-212206, S6-212403, S6-212483, S6-212401, S6-212484, S6-212486, S6-212456 | 0.3.0 |
| 2021-12 | SA6#46-BIS-e |  |  |  |  | S6-212720, S6-212730, S6-212748, S6-212796, S6-212807, S6-212813, S6-212829, S6-212830, S6-212831, S6-212832 | 0.4.0 |
| 2022-02 | SA6#47-e |  |  |  |  | S6-220077, S6-220457, S6-220458, S6-220459, S6-220460, S6-220340, S6-220461, S6-220274, S6-220420, S6-220462, S6-220479, S6-220299, S6-220102, S6-220463, S6-220464, S6-220465, S6-220466, S6-220468, S6-220480, S6-220469, S6-220470, S6-220349, S6-220454, S6-220455. | 0.5.0 |
| 2022-03 | SA6#47-e |  |  |  |  | Missing clause 7 from S6-220274 implemented. | 0.5.1 |
| 2022-04 | SA6#48-e |  |  |  |  | S6-220632, S6-220730, S6-220769, S6-220864, S6-220868, S6-220913, S6-220914, S6-220951, S6-220952, S6-220953, S6-220954, S6-220955, S6-220956, S6-220957, S6-220958, S6-220959, S6-220960, S6-220961, S6-220962, S6-220963, S6-220966, S6-220977, S6-220978. | 0.6.0 |
| 2022-05 | SA6#49-e |  |  |  |  | S6-221056, S6-221331, S6-221272, S6-221442, S6-221431, S6-221330, S6-221462, S6-221463, S6-221488, S6-221353, S6-221464, S6-221465, S6-221466, S6-221489, S6-221467, S6-221468, S6-221490, S6-221432, S6-221429, S6-221491, S6-221399, S6-221492, S6-221357, S6-221469, S6-221493, S6-221494, S6-221470, S6-221261, S6-221434, S6-221435, S6-221387, S6-221260, S6-221198 | 0.7.0 |
| 2022-05 | SA6#49-e |  |  |  |  | Implemented editorials related to S6-221429 and S6-221494. | 0.7.1 |
| 2022-06 | SA#96 | SP-220463 |  |  |  | Presentation for information at SA#96 | 1.0.0 |
| 2022-06 | SA#96 | SP-220684 |  |  |  | Implement missed changes as per S6-221432 and S6-221442. | 1.0.1 |
| 2022-07 | SA6#49-bis-e |  |  |  |  | S6-221883, S6-221969, S6-221885,S6-221970, S6-221971, S6-221887, S6-221862, S6-221799, S6-221972, S6-221973, S6-221974, S6-221975, S6-221937, S6-221938, S6-221939, S6-221996, S6-221940, S6-221524, S6-221525, S6-221976, S6-221977, S6-221978, S6-221889, S6-221997, S6-221775, S6-221801, S6-221979, S6-221983, S6-221980, S6-221793, S6-221981, S6-221982, S6-221627, S6-221672, S6-221984, S6-221935, S6-221985, S6-221986, S6-221620, S6-221988, S6-221989, S6-221688 | 1.1.0 |
| 2022-07 | SA6#49-bis-e |  |  |  |  | Reimplementation of S6-221980 | 1.1.1 |