| 3GPP TR 23.700-99 V0.3.0 (2021-10) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Network Slice Capability Exposure for Application Layer Enablement (NSCALE) (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	7

4	Application architecture for network slice capability exposure	8

4.1	Application layer support aspect requirements	8

4.1.1	General requirements	8

4.1.2	<title of requirements#1>	8

4.2	Application architecture	8

4.2.1	General	8

4.2.2	Architecture	8

4.2.3	Functional elements	10

4.2.3.1	Network slice Capability Exposure client	10

4.2.3.2	Network slice Capability Exposure server	10

5	Key issues	10

5.1	Key issue 1: Network slice capability management enhancements	10

5.2	Key issue 2: Application layer exposed network slice lifecycle management	10

5.3	Key issue 3: Discovery & registration aspects for management service exposure	11

5.4	Key issue 4: Network slice fault management capability	11

5.5	Key issue 5: Communication service management exposure	12

5.6	Key issue 6: Application layer QoS verification capability exposure	13

5.7	Key issue 7: Network slice related performance and analytics exposure	13

5.8	Key issue 8: Support for requirements translation	14

5.9	Key issue 9: Support for trust enablement	14

5.10	Key Issue 10: Support for managing trusted third-party owned application(s)	15

5.11	Key issue 11: Dynamic slice SLA alignment	15

6	Solutions	16

6.0	Mapping of Solutions to Key Issues	16

6.1	Solution 1: Automatic application layer network slice management	16

6.1.1	Solution description	16

6.1.1.1	General	16

6.1.1.2	Automatic application layer network slice lifecycle management	17

6.1.2	Solution evaluation	18

6.2	Solution 2: Network slice fault management capability	18

6.2.1	Solution description	18

6.2.1.1	General	18

6.2.1.2	Network slice fault management capability exposure	18

6.2.2	Solution evaluation	20

6.3	Solution 3: Slice API configuration and translation	20

6.3.1	Solution description	20

6.3.1.1	General	20

6.3.1.2	slice API configuration	20

6.3.1.3	Slice API translation	22

6.3.2	Solution evaluation	23

6.X	Solution #X: <title>	23

6.X.1	Solution description	23

6.X.2	Solution evaluation	23

7	Overall evaluation	23

8	Conclusions	23

Annex A: Change history	25

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The study identifies the SA1 requirements and potential vertical requirements of network slice capability exposure that have yet to be realized, and proposes application architecture aspects solutions and enhancements to SEAL. Based on any gaps found between requirements and existing capabilities, the present document includes: identification of key issues, solutions, corresponding evaluations and conclusions to ensure the efficient network slice capability exposure.

The study takes into consideration the SEAL architecture specified in 3GPP TS 23.434 [2],  network slicing related architecture and services(such as services of NEF) specified in 3GPP TS 23.501 [3] and 3GPP TS 23.502 [4], network slicing management capability provisioning and architecture specified in 3GPP TS 28.531 [5] and 3GPP TS 28.533 [6], requirements specified in TS 22.261 [7].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[3]	3GPP TS 23.501: "System architecture for the 5G System (5GS)"

[4]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[5]		3GPP TS 28.531: "Management and orchestration; Provisioning".

[6]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[7]	3GPP TS 22.261: "Service requirements for the 5G system".

[8]	3GPP TS 28.533: "Management and orchestration; Generic management services".

[9]	3GPP TR 22.835: " Study on enhanced access to and support of network slices".

[10]	3GPP TS 28.545: "Management and orchestration; Fault Supervision (FS) ".

[11]	3GPP TS 28.535: "Management and orchestration; Management services for communication service assurance; Requirements".

[12]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[13]	3GPP TS 28.550: "Management and orchestration; Performance assurance".

[14]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[15]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[16] 	3GPP TS 28.104: "Management and orchestration; Management Data Analytics".

[17]	3GPP TS 32.111-1: "Management and orchestration; Fault management, Part 1: 3G fault management requirements".

[18]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[19]				3GPP TS 28.104: "Management and orchestration; Management Data Analytics".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

Auto-NS-LCM	automatic application layer network slice lifecycle management

# 4	Application architecture for network slice capability exposure

## 4.1	Application layer support aspect requirements

### 4.1.1	General requirements

[AR.4.1.1-x]	The application enablement layer shall support interaction with 3GPP network management system to consume network slice management service.

NOTE: The consuming of the network slice management service related procedures are specified in TS 28.531[5].

Editor's Note:	The interface between the 3GPP network management system and the application enablement layer is in SA5's scope.

### 4.1.2	<title of requirements#1>

Editor's Note:	This subclause will describe the application layer support aspect requirements.

## 4.2	Application architecture

### 4.2.1	General

This clause provides the overall architecture description:

-	Clause 4.2.2 describes the application architecture in the service based representation and reference point representation;

-	Clause 4.2.3 describes the functional entities;

### 4.2.2	Architecture

Figure 4.2.2-1 Architecture for network slice capability exposure – Service based representation

Figure 4.2.2-1 exhibits the service-based interfaces for providing and consuming network slice capability exposure services.

The mechanisms for service discovery in the service-based representation depicted in figure 4.2.2-1 are as follows:

-	The network slice capability exposure server could provide service to VAL server and NSCE client through interface SNsce.

NOTE: the NSCE layer is the enhancement of SEAL NSCM layer, but which term is going to be used in the specification is FFS.

Figure 4.2.2-2illustrates the service-based representation for utilization of the 5GS network services based on the 5GS SBA specified in 3GPP TS 23.501 [3].

Figure 4.2.2-2 Architecture for network slice capability exposure utilizing the 5GS network services based on the 5GS SBA – Service based representation

Figure 4.2.2-3 depicts the network slice capability exposure architecture in the non-roaming case, using the reference point representation showing how various entities interact with each other.

Figure 4.2.2-3 Architecture for network slice capability exposure – reference points representation

The network slice capability exposure client communicates with the network slice capability exposure server over the NSCE-UU reference point. The network slice capability exposure client provides the support for network slice capability exposure functions to the VAL client(s) over NSCE-C reference point. The VAL server(s) communicates with the network slice capability exposure server over the NSCE-S reference point. The network slice capability exposure server, acting as AF, may communicate with the 5G Core Network functions via NEF (N33) reference point (for interactions with PCF, NSACF, etc.), or interacting with PCF directly via N5, if permitted. The network slice capability exposure server may interact with OAM system over NSCE-OAM reference point, as consumer in both NSaaS and NoP model (for Network Slice Provisioning capabilities, Performance Assurance,   Fault Supervision etc.).

Editor's Note:	The NSCE-OAM reference point is not specified yet, and the entities the network slice capability server interacts with in the management plane is being studied in the SA5.

### 4.2.3	Functional elements

#### 4.2.3.1	Network slice Capability Exposure client

The network slice capability exposure client functional entity acts as the application client for the slice enablement. The network slice capability exposure client interacts with the network slice capability exposure server to trigger a network slice related operations. This trigger may be due to an application QoS requirement change, a service operation change, a network slice status change, etc. The NSCE client may receive a network slice related notification from the NSCE server. The NSCE client may optionally notify the VAL client on the network slice / DNN change.

#### 4.2.3.2	Network slice Capability Exposure server

The network slice capability exposure server functional entity provides application layer enablement to support the network slice management and control with/without invoking control and management plane capabilities from SA2 and SA5 pertaining to network slicing. Such enablement supports the network slice related operations such as the mapping or migration of one or more vertical applications to one or more network slices, triggering the dynamic network slice lifecycle management, NSI/NSSI monitoring, etc.

# 5	Key issues

## 5.1	Key issue 1: Network slice capability management enhancements

SEAL is the service enabler architecture layer common to all vertical applications over 3GPP systems. It provides the functions like location management, group management, configuration management, identity management, key management, network resource management and network slice capability management as defined in 3GPP TS 23.434 [2].

Network slicing is a general network capability which can be applied for many vertical industries. The network slice capability management service in 3GPP TS 23.434 [2] only provides capability of network slice adaptation between NSCM server and the NSCM client. However, for such capability both control plane and management plane interactions need to be considered since there is close coupling between the per UE session and the per slice related actions. For example, the SEAL/NSCM layer may need to be aware of the slice provisioning parameters (NSI/NSSI configuration) which can be provided by the slice management system. Furthermore, NSI/NSSI performance monitoring from management system (e.g. NSI/ NSSI status) may be useful to be known at the enabler layer, since this may affect the application to slice re-mapping triggering (e.g. to re-map to the least congested slice); and may also impact other SEAL provided functionalities (e.g. QoS/resource control, group management etc).

Open issues:

-	Whether and which enhancement to SEAL network slice capability management service is required having in mind SA2 and SA5 slice related exposure?

-	Whether a potential enhancement to SEAL network slice capability management functional model is required?

-	Whether new or enhanced APIs are needed to support the potential SEAL enhancements?

-	How the network slice capability (such as management service (MnS)) consumption may trigger and impact the value-add services provided by the SEAL layer?

## 5.2	Key issue 2: Application layer exposed network slice lifecycle management

It is specified in clause 6.10 of 3GPP TS 22.261 [7] that 5G network is required to provide suitable APIs to allow a trusted third-party to create, modify, scale and delete network slices used for the third-party. As specified in 28.531 [5], SA5 has defined the network slice lifecycle management service. SA6 can provide a more concise application layer exposed network slice lifecycle management with additional functionality for verticals. It can help vertical do the lifecycle management without knowledge of SA5 O&M defined interfaces and APIs. In addition, with the network slice related information (such as network slice status reported from NSACF), the SA6 could trigger some slice lifecycle management operations automatically to provide a value-added lifecycle management service. It is not clear how to expose the network slice lifecycle management service from application perspective with providing additional value on top of existing SA5 solution.

Editor's note: The interface which may be used in this KI specified in TS 28.532 [8] is still under the discussion of SA5, and whether the enabler layer could use the interface directly or act as an entity to trigger the relevant procedures depends on SA5's decision.

Hence, it is required to study the following issues:

-	How to integrate and expose the SA5 and SA2 network slice related capability to provide some value-added lifecycle management service from application layer?

-	Whether and how additional service APIs are required to be supported for application layer exposure of network slice lifecycle management?

-	Whether and how CAPIF can be leveraged for additional service APIs.

## 5.3	Key issue 3: Discovery & registration aspects for management service exposure

There are use cases (being discussed also in SA1, TR 22.835 [9]), where the applications (e.g. gaming or online video applications) may access the 5GS over multiple slices for different services (e.g. based on the user membership); or have different priorities on different slices based on Application Service Provider (ASP) request. As an example, a mobile network operator has provisioned a set of network slices (Slice#1, Slice#2, Slice#3) which may be used by different ASPs (e.g. Slice#1 for online video services, Slice#2 for gaming.  Slice#3 for eMBB or IOT service). Different ASPs may use these slices (or a subset of them) for different services that they offer. Furthermore, when an application changes the network slices to be accessed, it should be agnostic to the UEs accessing the service and should be performed automatically.

The vertical enablement layer (SEAL, vertical-specific enablers) supports the exposure of telco provided services to the vertical / application service provider (ASP). Such telco-provided services traditionally covered the 3GPP control plane services (provided by SA2); however, these can be extended to 3GPP management domain services (provided by SA5) which may be useful for allowing the ASP to monitor and manage the slices used by the UEs. The enabler layer can be seen as a trusted application entity that interacts with the management system on behalf of ASP to allow the exposure of management services related to the offered slices. It can be used to trigger dynamic slice-related actions and also reduce complexity at the ASP side.

In this extended notion of vertical enablement (to cover the 3GPP management service exposure), the vertical/ slice customer needs to be able to 1) discover the relevant Network Slice Instances and the respective capabilities such as coverage offered, RAT/ frequencies, to 2) discover the management services (based on TS 28.533 [6]) which can be exposed as part of the offered slices, and 3) register to the 3GPP management domain via the vertical enabler layer for consuming the management services.

This study needs to investigate:

-	Whether and how SEAL needs to be enhanced to support the discovery of the offered slices and the offered management services related to these slices, to the vertical/ASP;

-	Whether and how SEAL needs to be enhanced to support the registration of the vertical applications to the 3GPP management domain.

## 5.4	Key issue 4: Network slice fault management capability

As the requirement captured in TS 22.261 [7], based on operator policy, the 5GS shall provide suitable APIs to allow a trusted third-party to monitor the network slice used for the third-party according to operator policy.

In addition, the descriptions of network diagnostics are captured in clause F.2, TS 22.261 [7] as following:

Network diagnostics helps with scanning, diagnosing and identifying problems within a network. Diagnostics includes gathering data and continuously providing sufficient performance parameters that characterize the quality of the network connection. This includes data of the physical connection as well as of logical links and sub-networks. Exposure of relevant (and possibly aggregated) performance parameters ensures a quick reaction in case of failure as well as identifying network connectivity, performance and other related problems.

Network diagnostic information needs to be generated automatically and, in case of a hosted or virtual network deployment, be made available to the tenant of the network via a suitable API.

The alarm data can be used to help the third-party to diagnose the fault problem of the services, locate the fault causes, and to be aware of the potential fault. In TS 28.545 [10], the fault supervision management services are standardized by which the alarm of the network slice instance from network resource aspects can be subscribed and reported. This alarm information together with the application function's fault report and communication service related knowledge can be utilized by the SEAL/NSCM to diagnose and locate the cause of the service performance deterioration and the fault of the communication services, and then exposed the fault report to the third-party. For example, if the status of the required communication is not correct, the SEAL/NSCM derives this alarm information from application functions. In this case, it is the SEAL/NSCM's responsibility to detect whether this fault is caused by the 5GS network or not and exposed the fault report to the third-party. If it is, then the SEAL/NSCM may inform the management functions the location of the fault and ask for the maintenance of the managed functions to clear the fault.

In addition, the two business relationships of network slice as a service and network slice as NOP internals may considered separately for the capabilities exposed in each scenarios. The coordination with SA5 is needed to fill the gap between the communication services (from the verticals' point of view) and the 5G network.

Open issues:

-	Whether and how additional APIs dedicated to network slice fault management capability are required?

-	How to define the APIs to expose the fault report to verticals client/server

-	How to coordinate with fault supervision management services provided by O&M systems defined in SA5 to fill the gap in fault management.

## 5.5	Key issue 5: Communication service management exposure

In clause 6.10 of TS 22.261 [7], following requirements are defined:

Based on operator policy, the 5G network shall expose a suitable API to allow an authorized third-party to define and reconfigure the properties of the communication services offered to the third-party.

The 5G system shall support the means for disengagement (tear down) of communication services by an authorized third-party

In addition, in clause 6.23.1 of TS 22.261 [7], it defines that QoS monitoring can be used for assessing and assuring the dependability of the communication services.

In SA5 TS 28.535 [11], management services to assure the communication service as per agreement (for example a SLS) with a communication service (only network slice as a service scenario) consumer (e.g. enterprise) have been defined. For other scenarios, the communication services from the verticals aspects required by SA1 (e.g., vertical automation communication services, URLLC services) are not discussed in SA5

From the vertical industry perspective, they may be more concerned about how to operate the communication services provided by 5GS to meet the business requirements on application level. Take V2X service as an example, there is a specific service of cooperative driving for vehicle platooning information exchange, and the vertical has some requirements on this service, including end-to-end latency between two UEs. Therefore, the service enable layer needs to provide such capabilities for vertical industries, including lifecycle management and quality assurance of communication services from the verticals' perspective. For example, with the QoE/QoS data of communication services collected from vertical application layer, SA6 may need to provide some value-added communication services management and/or assurance services.

It is not clear how to expose the communication service management service from application perspective with providing additional value on top of existing SA5 solution.

Hence, it is required to study the following issues:

-	What kinds of service APIs are required to be supported for application layer exposure of communication services life cycle management?

-	What kinds of additional service APIs are required to be supported for application layer exposure of communication services SLA assurance?

-	How to support above capabilities in the enable layer?

-	How to coordinate with SA5 functionalities to fill the gap between the communication services (from the verticals' point of view) and the 5G network.

## 5.6	Key issue 6: Application layer QoS verification capability exposure

In clause 6.23 of TS 22.261 [7], it is defined the requirements about QoS monitoring that 5G system shall be able to assessing and assuring the dependability of the communication services.

In clause F.1 of TS 22.261 [7], it is discussed how QoS monitoring information can be used for assurance purposes. In step "Customer rating of QoS", it mentions that the customer can compare the QoS achieved by the provider with the QoS requirements and its own experience of the QoS.

In clause SA5 TS 28.552 [12], it defined QoS measurements reports with different Filters from perspective of OAM, e.g. 5QI, QIC, S-NSSAI, PLMN.

In some cases, the verticals and the service provider reach an agreement on the SLA, however with this SLA requirement, the VAL client may also suffer unsatisfied experience. Hence, it could be possible to provide Vertical application layer the capability of comparing the QoS achievement status together with the OAM QoS data versus real customer QoS data (e.g., MOS) collected from VAL client to check whether the existing QoS data is able to satisfy the VAL client's.

It is not clear how to expose the QoS verification capabilities on top of existing SA5 solution.

Hence, it is required to study the following issues:

-	What kinds of additional service APIs are required to be supported for application layer exposure of QoS verification?

-	What kinds of additional service APIs are required to obtain the real vertical QoS to support the QoS verification?

-	How to support above capabilities in the enable layer?

## 5.7	Key issue 7: Network slice related performance and analytics exposure

As specified in clause 6.10 of 3GPP TS 22.261 [7], 5G network is requested to support a 3rd party to get the network status information of a private slice dedicated for the 3rd party. The 5G network collects various kinds of data, such as performance measurements in TS 28.552 [12] and analytics data as specified in 3GPP TS 28.104 [16] and the analytics data from NWDAF and NSACF exposed by NEF. However, to get the information efficiently, the verticals are supposed to know, which entity to interact with to get the desired information, how to extract useful information from data which is collected using different statistical methods from different entities, which is challenging for some verticals. The network slice capability exposure enabler layer can aggregate and process the data from different source, making the network information exposure more orderly and easier to read. For example, for the slice related performance and analytics come from multiple sources, the enabler layer could help to organize and aggregate the information. For some applications utilizing multiple S-NSSAI, the enabler layer could help to organize and aggregate the information so that it is exposed and displayed based on the application level rather than slices level.

Editor's note: Whether this SID is going to study the interface which is used to get the performance measurements and analytics is FFS.

Hence, it is required to study the following:

-	Whether and how information about available (SA2 and SA5) slice performance and analytics should be exposed to a third party with added value?

-	Whether and how available slice performance and analytics related information could be aggregated or processed to support an efficient information exposure?

-	Whether and how additional service APIs are required to be supported at SEAL for the network slice measurements and analytics exposure?

-	Whether and how CAPIF can be leveraged for additional service APIs?

## 5.8	Key issue 8: Support for requirements translation

Requirements for network slice from different verticals may vary from one to another, in terms of performance and capability requirements. For example, for the performance related requirements, the live video streaming cares more on bandwidth while V2X cares more on latency and jitters. For the capabilities related requirements, V2X may requests the positioning while future factory may request the self-control and management. To satisfy the requirements, vertical has to interact with several 5GS entities and understand the specific network parameters, such as the attributes in the service profile as defined in TS 28.541 [14]. Slice enabler layer could act as a mediator between the vertical customer and the 5GS to decompose and translate the requirements.

However, to cope with the various requirements, it is needed to specify the operations and procedure on how to translate them more efficiently, such as in the unified manner with unified format/model/template. In such way, the slice enabler layer is capable of translating requirements from the vertical, to service consumption or service API invocation and configurations to the respective 5GS domains. The service could be provided from control plane, management plane and SA6 slice enabler layer itself, or the combinations of services above.

There are two main aspects in requirements translation: 1) one is how the vertical requirements could be collected, whether and how the template is needed? 2) The other is how to transfer the requirements into actions, whether and how API translation is needed?

This key issue aims to discuss how to configure and translate the requirements to service consumption and configuration in a way that the slice capability exposure is 1) agnostic to the underlying telecom infrastructure, 2) hides the complexity of telecom infrastructure, 3) doesn't impact/restrict the level of exposure to the vertical and 4) that is resilient to dynamic changes that may happen due to application portability or telco-provided API status changes.

Therefore, the open issues include:

-	Whether and how the vertical requirements could be collected at the slice enabler to allow the translation to network service consumption and configuration,

-	Whether and how API translation is needed to support the requirements translation.

## 5.9	Key issue 9: Support for trust enablement

A vertical application may use the slice enabler services (which can be seen as a trusted 3rd party to the MNO) to request management services as well as control plane services for a new slice on demand, based on an agreement between the vertical and the network slice provider.

However, the creation of a new slice will require a form of trust between the vertical/end application and the 5GS (management and control plane) for authorizing/authenticating the application request and enabling the vertical app to consume management / control services related to the requested slice.

The vertical application may not be trusted by the OAM or the 5GC, and is therefore not able to access management and control services. Also, there can be two way of accessing these services after authorization, via direct exposure or indirectly via the enabler server.

So, the key issue will study:

-	How to enable the authorization/authentication of the vertical application to consume telco-provided services (management and control plane), based on the vertical applications request.

- 	How to enable the authorization/authentication of the vertical application to consume telco-provided services (management and control plane) indirectly via the slice enabler layer, based on the vertical applications request.

## 5.10	Key Issue 10: Support for managing trusted third-party owned application(s)

As per 3GPP TS 22.261 [7], it is possible for trusted third-party to use a dedicated network slice for diverse use cases. Further, it provides following requirement to manage applications:

"Based on operator policy, a 5G network shall provide suitable APIs to allow a trusted third-party to manage this trusted third-party owned application(s) in the operator's Service Hosting Environment."

It is also possible for the third-party to offer its consumers different contract qualities level (e.g. gold, silver and bronze). In clause 5.7.1 of 3GPP TR 22.835 [9], following use case has been specified:

"For gaming or online video applications, the end users, who have subscription with MNOs who may provide multiple network slices to different users or services, may still have different priority or membership e.g. VIP maintained by 3rd party Service Provider (SP). And depending on the priority or membership information from 3rd party SP perspective, based on the agreement between SP and MNO, the UE have different priority for the available network slices."

In clause 4.2.11.2 of 3GPP TS 23.502 [4] specifies following:

"When for all the Requested S-NSSAI(s) provided in step 2 the NSACF returned the maximum number of UEs per network slice has been reached and if one or more subscribed S-NSSAIs are marked as default in the subscription data and not subject to Network Slice Admission Control, the AMF can decide to include these Default Subscribed S-NSSAIs in the Allowed NSSAI. Otherwise, the AMF rejects the UE request for registration. In the Registration Reject message the AMF includes the rejected S-NSSAI(s) in the rejected NSSAI parameter, and for each rejected S-NSSAI the AMF includes a reject cause to indicate that the maximum number of UEs per network slice has been reached and optionally a back-off timer."

Upon reaching maximum UEs slice quota, the 5GC may reject the registration request on the S-NSSAI from the gold quality level customer which may not be desirable by the trusted third party. This happens since 5GC is not aware of the relevant application information e.g. contract qualities level of the UE making the registration request.

The third party application needs to provide high priority to the higher level of contract qualities and so it needs to manage such connections.

In particular, this KI will address:

-	Whether and how the AF can provide application policy information to the operator's Service Hosting Environment to enable automatic management of application resources?

-	What application policy information needs to be specified by the trusted third-party AF that can be automatically evaluated by the operator's Service Hosting Environment?

-	Whether and how the AF can manage use of the application resources in the operator's Service Hosting Environment on a per user characteristic?

## 5.11	Key issue 11: Dynamic slice SLA alignment

Some verticals/enterprises have strong desire for the slice/service self-management. Initially, they may order a slice with certain NEST (Network Slice Type) where the SLA parameters are already provided based on the static and estimated service requirements. After the service is executed on the slice, the slice may not fully match the service real-time running conditions, for example, maybe only 60% of the slice resource is used to support the service, and rest of the slice resource is always idle, or the slice resource is insufficient due to under-provisioning. In order to achieve the maximum return of investment and slice/service self-management, the verticals/enterprises expect a more optimal service profile which contains the exact value of those SLA parameters to support to the executing services' demands, i.e., dynamic SLA alignment.

However, the verticals/enterprises may not fully understand the slice SLA parameters, or they may not set proper SLA parameters initially. How to enable better alignment between the vertical needs and the SLA parameters needs to be studied.

NOTE: All the potential changes to the slice are based on existing SA2 and SA5 service/capability.

Open issues:

-	Whether and How the SEAL network slice capability management service supports better alignment between vertical needs and SLA parameters for initial service requirements and ongoing service conditions?

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key issue 1 | Key issue 2 | Key issue 3 | Key issue 4 | Key issue 5 | Key issue 6 | Key issue 7 | Key issue 8 | Key issue 9 | Key issue 10 | Key issue 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solution 1 |  | X |  |  |  |  |  |  |  |  |  |
| Solution 2 |  |  |  | X |  |  |  |  |  |  |  |
| Solution 3 |  |  |  |  |  |  |  | X |  |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |

## 6.1	Solution 1: Automatic application layer network slice management

Editor's Note:	Provide a suitable title for the solution.

### 6.1.1	Solution description

#### 6.1.1.1	General

This solution aims to address the issues identified in Key Issue 2.

This solution provide an procedure for automatic application layer network slice management performed by the network slice capability exposure server based on network slice status collected from 5GS and QoE collected from application layer.

When network slice capability exposure server receives a request for automatic application layer network slice management from VAL server, the network slice capability exposure server performs the service operations including detecting and subscribing the event which may trigger the automatic network slice lifecycle management, making the network slice lifecycle management recommendation/decision, triggering the network slice management operations, notifying the consumer about the network slice information.

#### 6.1.1.2	Automatic application layer network slice lifecycle management

Figure 6.1.1.2-1 illustrates an automatic application layer network slice lifecycle management solution based on network slice related data and QoE collected from application layer.

Pre-conditions:

1.	The VAL client has requested a network slice provisioning;

2.	The VAL server has subscribed to the network slice capability exposure server for automatic network slice management;

3.	The network slice enabler layer is registered/capable for interacting with 5GS such as triggering network slice LCM operations, and has collected current network slice capabilities.

Figure 6.1.1.2-1: Automatic application layer network slice lifecycle management

1.	The vertical server sends a request for automatic application layer network slice lifecycle management (auto-NS-LCM), with network slice requirements (e.g. delay, throughput, load, the maximum number of users supported, etc.). The request can indicate the level of auto-NS-LCM that is required, such as whether to notify the VAL server/consumer before performing the auto-NS-LCM. The request may also indicate the trigger conditions, such as by providing the monitored parameters and the corresponding thresholds.

2.	After receiving the request, the network slice capability exposure server checks that the user is authenticated and authorized to perform the corresponding auto-NS-LCM operations, and filters the unauthorized requests, if any.

3.	If authenticated and authorized, the network slice capability exposure server, acting as the network slice LCM service consumer, triggers the AllocateNsi request (see TS 28.531 clause 6.5.1 [5]) towards the respective management service provider, based on the network slice capabilities and network slice requirements.

4.	The network slice capability exposure server sends the auto-NS-LCM response to the VAL server with the result(s) of the network slice LCM operation(s).

5.	According to network slice requirements, network slice capability exposure server triggers the provision of network slice status and QoE metrics.

5a.	The network slice status could be collected through subscribing or requesting to 5GS. For example, to monitor the slice load, it could subscribe to/request the relevant service(s), such as AnalyticsExposure defined in TS 23.502 [4] clause 5.2.6.16, provisioning data report exposure for NSI in clause 5 of TS 28.532 [15]. If the trigger conditions are not indicated in the subscription, the network slice capability exposure server may help to configure an appropriate trigger condition, such as report period or thresholds.

5b.	Also, the network slice capability exposure server could get the information of QoE metrics from the application layer domain.

6.	Once the trigger condition or a combination of trigger conditions are met, based on requirements and network slice capabilities with updated information in Step 5, such as change in QoE, change in network slice status etc., the network slice capability exposure server determines whether and what network slice LCM operations should be taken and makes the decision(s)/recommendation(s), such as modifyNsi/AllocateNsi/DeallocateNsi request as specified in TS 28.531 [5].

7.	Optionally, if it is indicated in the request to notify the VAL server/consumer before performing the auto-NS-LCM, the network slice capability exposure server sends the network slice LCM recommendation(s) with network slice status to VAL server, to see whether takes the recommendation(s) or not.

8.	Optionally, sending the response indicating the decision made by VAL server to network slice capability exposure server.

9.	Based on decision made by VAL server or network slice capability exposure server, the network slice capability exposure server performs the corresponding operation(s).

10. According to the corresponding operation(s) result, the network slice capability exposure server sends the response to the VAL server.

### 6.1.2	Solution evaluation

Editor's Note:	This subclause will evaluate the solution.

## 6.2	Solution 2: Network slice fault management capability

### 6.2.1	Solution description

#### 6.2.1.1	General

This solution addresses the key issue 4 of network slice fault management described in clause 5.4.

This solution provides a possible procedure to illustrate the network slice fault management capability exposed by NSCE server.

To enable the VAL server to monitor the problems of a network. The NSCE layer should help the VAL server to scanning, diagnosing and identifying problems within a network. The performance data and alarm data from multiple sources is helpful to characterize the quality of the network connection.

#### 6.2.1.2	Network slice fault management capability exposure

Figure 6.2.1.2-1 illustrates the network slice fault management process to address the key issue 4 of network slice fault management described in clause 5.4.

Pre-conditions:

1.	The VAL client has subscribed to the network slice fault management capability.

2.	The network slice enabler layer is capable to interact with NEF and OAM system.

3.	The VAL client has checked the status of application layer.

Figure 6.2.1.2-1 Network slice fault management process

1.	The VAL server sends a request to NSCE server to request for the fault diagnosis of the applications and networks when it detected a fault in the application layer, the services and the network slices. This request may be triggered by the errors of the applications detected by the VAL server itself, or the VAL server may periodically collect the fault diagnostic report subscribing to the network slice fault management capability. The fault information of applications may be included in this request. For example, the Service Availability Failure Events e.g, described as Fallback events carried out by the Dual SIM cellular device.

2.	On receiving the request from VAL server, NSCE server retrieves the performance data of network slice from 5GS. For OAM system, the APIs defined in clause 11.3, TS 28.532[15] is utilized. For CN functions, the APIs of Nnwdaf_AnalyticsInfo service defined in clause 7.3, TS 23.288[18] can be utilized.

The analytics data defined in clause 6.3 to clause 6.14, TS 23.288[18], network slice instance related performance data defined in clause 5, TS 28.552[12] and network analytics data in TS 28.104[19] exposed by OAM system may be acquired.

Editor's Note: The analytics data in TS 28.104[19] is expected to be defined by SA5.

3.	NSCE server retrieves the alarms of network slice instances from OAM system via the procedures defined in clause 6.1,TS 28.545[10], and the alarms are defined in clause 4.1.1.1, TS 32.111-1 [17], e.g., the fault of communication, environmental, equipment, processing error, QoS for device/resource/file/functionality/smallest.

4.	NSCE server may request the alarm information (e.g., the 5GS network is not work or the required performance is under the threshold which leads the service's problem) collected by NSCE client if possible. The information collected by the NSCE client depends on the third-parties' requirements and implementation.

Editor's Note: The APIs utilized to achieve the fault data from application layer is to be defined in SA6.

5.	NSCE client report the requested fault information to NSCE server.

6.	NSCE server correlates the applications, services and the network resources of the specific network slice instances which supports the applications and services with the S-NSSAI, NSI ID (nSInstanceId defined in TS 28.541[14]), Application Identifier (defined in 23.501[3]), and then diagnoses the causes of the fault of the applications or services by analysing the fault information from different sources. For example, the RAN function of the slice instance which is utilized to support the service of the smart grid application, for a certain time duration, the smart grid suffered the bad experience caused by Service Availability Failure Events, and in the RAN function is detected continuously to report an alarm of environmental fault in the same time duration, then the environment fault may be the root cause of the Service Availability Failure and should be prioritized to be solved.

Editor's Note: Clarification of what prioritization means is FFS

7.	NSCE server response to VAL server with the reports of fault diagnostic result to indicate the causes of the failures of the applications or services. If there is no problems within the 5GS network, the VAL server may be requested to check the application layer itself again.

8.	If the NSCE server detects that the application/service error is caused by the 5GS, the VAL server may send the fault diagnosis report to OAM system to indicate the server fault which causes the application/service failure by utilizing the NSCE-OAM interface.

Editor's Note: The APIs utilized to send the fault diagnosis report to OAM is to be defined in SA6.

### 6.2.2	Solution evaluation

Editor's Note:	This subclause will evaluate the solution.

## 6.3	Solution 3: Slice API configuration and translation

### 6.3.1	Solution description

#### 6.3.1.1	General

This solution aims to address the issues identified in Key Issue 8.

This contribution provides a solution for the API configuration and translation and proposes the translation of the service API as invoked by the end applications to slice APIs based on the API configuration and application to slice mapping. Slice APIs can be defined as customized/tailored sets of service APIs (which can be either NEF northbound APIs or OAM provided APIs or enabler layer/SEAL provided APIs) and can be mapped to particular slice instances. The slice APIs can be a bundled or combined API comprising of different types of APIs, which will be used to expose the telco (5GS/SEAL)- provided services as needed by the applications of the slice customer. Each slice API may be configured per network slice instance.

This solution provides a support functionality to the vertical application specific layer and configures the exposure of APIs in a slice-tailored manner. It is assumed that the VAL server is not initially aware of the all the API exposure capabilities and information which will be needed for the given slice based on the SLA, and NSCE plays a vital role in configuring and translating the slice API based on the per slice requirements to service APIs.

This solution has two procedures:

-	a procedure on the slice API configuration

-	a procedure on the slice API translation

#### 6.3.1.2	slice API configuration

In this procedure, the VAL server initially provides an application requirement to enabler server including the service KPIs and the subscribed/preferred slices. Then, the slice enabler configures the mapping of the VAL application to a slice API which is a combination/bundling of northbound APIs (from both management and control plane). In particular, a slice API (or SDK) consists of telco-provided/platform dependent service APIs (e.g. NEF, OAM, SEAl, etc), and provides an abstraction/simplification on top of them. The procedure also covers the scenario where a trigger event occurs (e.g. QoS degradation, slice load) and the mapping configuration or the slice API configuration needs to change. In this scenario, the slice enabler updates the configuration of the API and provides a notification to the VAL server.

Figure 6.3.1.2-1 illustrates a solution for the slice API configuration.

Pre-conditions:

1.	The VAL server has registered to receive network slice capability exposure services

Figure 6.3.1.2-1: slice API translation

1.	The VAL server sends a VAL application requirement request to the network slice capability exposure server. This request provides the service requirements / KPIs, the capability exposure requirements and a preferred/subscribed slice identification (e.g. S-NSSAI or ENSI)

2.	The network slice capability exposure server maps the VAL application requirement to a slice API which includes a list of APIs which are needed to be consumed as part of this service capability exposure (such APIs can be NEF APIs as specified in TS 29.522 clause 5 e.g. related to network monitoring, slice status, analytics exposure, SEAL APIs as specified in TS 23.434, OAM provided APIs). Such mapping can be determined at the network slice capability exposure server based on the VAL application exposure requirements or can be pre-configured per slice instance. The criteria for the mapping are the capability exposure requirement per slice (based on GST parameters, or from service/slice profile] as well as the capability exposure permissions/authorization for the API invoker.

The network slice capability exposure server may also store the mapping of the slice API to the service API list and per service API information (e.g. data encoding, transport technology, API protocol and versions)

Editor's Note: The exposure of MnS via OAM-provided APIs requires coordination with SA5 and is FFS

3.	The network slice capability exposure server subscribes/registers to consume the corresponding APIs from the 5GS (NEF and OAM) and SEAL service producers. For example, network slice capability exposure server may subscribe to consume NEF monitoring events, or SLA monitoring from OAM.

4.	The network slice capability exposure server sends a VAL application requirement response to notify on the result of the request and indicate whether configuration of the slice API can be possible or not.

5.	The network slice capability exposure server sends the slice API information and optionally the slice to service API mapping to the VAL server.

6.	A trigger event is captured by the service/API providers (e.g. control plane, management plane, SEAL) or the VAL server side (application server relocation to different EDN/DN, UE mobility to different EDN, application change of behavior) or any other API related event captured by the network slice capability exposure server (e.g. failure, unavailability, high load).

7.	The network slice capability exposure server processes the trigger event and checks and updates the mapping of service APIs to the slice APIs. The objective is to keep the slice APIs unchanged, so the VAL server is not aware of any change (if not triggered by VAL server). To accomplish this, the service APIs may need to be updated accordingly. For example, if one service API changes (e.g. due to high load, unavailability,  ..) the mapping to service APIs should be updated (e.g. if service API is a Location API which is provided by 5GC in the first place, the update would trigger the remapping to a Location API from SEAL LMS) to avoid affecting the slice API.

8.	The network slice capability exposure server updates the subscription/registration to the underlying 5GS and SEAL service producers, if an update on the service APIs (e.g. NEF APIs, SEAL APIs, OAM provided APIs) is needed.

9.	The network slice capability exposure server optionally notifies the VAL server on the slice/service API related updates.

#### 6.3.1.3	Slice API translation

This procedure follows the 6.3.1.2 and aims to describe how the slice API invocation request is translated to service API invocations after the slice API configuration mapping. In this procedure, the network slice capability exposure server initially receives a slice API invocation request from the vertical application based on step 5 of 6.3.1.2. Then, the network slice capability exposure server based on the slice API request fetches the service APIs to be invoked based on the slice API configuration and performs invocation requests to the corresponding service API providers.

As example, a slice API is requested for an IIOT slice. This may translate to: NSI Monitoring from Management Domain #1, NSSI Monitoring from Management Domain #2, network/QoS monitoring from NEF#1, Location monitoring from SEAL LMS, slice-related analytics from NWDAF (via NEF). Such translation could be based on the slice API configuration mapping of 6.3.1.2.

Figure 6.3.1.3-1 illustrates a solution for the slice API translation based on the configuration.

Pre-conditions:

1.	The VAL server has registered to receive network slice capability exposure services.

2.	The slice API mapping to the VAL server has been performed based on 6.3.1.2 step 2 and the slice API information is provided to the VAL server based on 6.3.1.2 step 5.

Figure 6.3.1.3-1: slice API translation

1.	The VAL server sends a slice API invocation request to the network slice capability exposure server. This request provides slice API information (name, type, communication methods, protocols,..),  based on the received information in step 5 of 6.3.1.2. Service APIs are indicated above can be NEF APIs as specified in TS 29.522, SEAL APIs as specified in TS 23.434, OAM provided APIs)

Editor's Note: The exposure of MnS via OAM-provided APIs requires coordination with SA5 and is FFS

2.	After receiving the request, the network slice capability exposure server checks that the user is authenticated and authorized to perform the slice API invocation and maps the requested slice API to a service APIs (based on the VAL server ID / slice ID and procedure in clause 6.x.1.2).

NOTE 1: If CAPIF is used, the network slice capability exposure server acts as AEF, and the authorization is obtained by CCF.

3.  The network slice capability exposure server generates a trigger for service API invocation requests to all the service APIs within the slice API.

4.	The network slice capability exposure server sends a service API invocation request to the producers of the service APIs within the slice API. This may include the originated service API invoker identity information (VAL server) as well as the direct API invoker information, authorization information, service API and slice API identification.

5.	The service API provider authenticates the API invoker and authorizes the request.

NOTE 2: If CAPIF is used, the requests are sent to the corresponding AEFs of the API provider's domain, and the authorization is obtained by CCF.

6.  The network slice capability exposure server receives the service API invocation response as a result of the service API invocation.

7. The network slice capability exposure server sends to the VAL server a slice API invocation response as a result of the slice API invocation.

### 6.3.2	Solution evaluation

Editor's Note:	This subclause will evaluate the solution.

## 6.X	Solution #X: <title>

Editor's Note:	Provide a suitable title for the solution.

### 6.X.1	Solution description

Editor's Note:	This subclause will describe the solution.

### 6.X.2	Solution evaluation

Editor's Note:	This subclause will evaluate the solution.

# 7	Overall evaluation

Editor's Note:	This clause will provide evaluation of different solutions.

# 8	Conclusions

Editor's Note:	This clause is intended to list conclusions that have been agreed during the course of the study item activities.

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-07 | SA6#44 |  |  |  |  | TR skeleton | 0.0.0 |
| 2021-07 | SA6#44 |  |  |  |  | TR Skeleton agreed in SA6#44: S6-211848 Implemented pCRs approved in SA6#44: S6-211849，S6-211850，S6-211851，S6-211782 Editorial changes by the rapporteur | 0.1.0 |
| 2021-09 | SA6#45 |  |  |  |  | Implemented pCRs approved in SA6#45: S6-212161，S6-212162，S6-212080，S6-212079，S6-211949，S6-211965，S6-211967，S6-212163，S6-212144，S6-212069 Editorial changes by the rapporteur | 0.2.0 |
| 2021-10 | SA6#45-bis |  |  |  |  | Implemented pCRs approved in SA6#45-bis: S6-212202, S6-212204, S6-212207, S6-212210, S6-212474, S6-212400 Editorial changes by the rapporteur | 0.3.0 |