| 3GPP TR 23.700-99 V0.5.0 (2022-02) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Network Slice Capability Exposure for Application Layer Enablement (NSCALE) (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

# Contents

Foreword	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	7

4	Application architecture for network slice capability enablement	8

4.1	Architectural requirements	8

4.1.1	General requirements	8

4.1.2	Security requirements	8

4.1.3	Registration requirement	8

4.1.4	Discovery requirement	8

4.1.X	<title of requirements#X>	9

4.2	Application architecture	9

4.2.1	General	9

4.2.2	Architecture	9

4.2.3	Functional elements	10

4.2.3.1	Network slice Capability Enablement client	10

4.2.3.2	Network slice Capability Enablement server	11

4.2.4	Service-based interfaces	11

4.2.5	Reference points description	11

4.2.5.1	VAL-UU	11

4.2.5.2	NSCE-UU	11

4.2.5.3	NSCE-C	11

4.2.5.4	NSCE-S	11

5	Key issues	12

5.1	Key issue 1: Network slice capability management enhancements	12

5.2	Key issue 2: Application layer exposed network slice lifecycle management	12

5.3	Key issue 3: Discovery & registration aspects for management service exposure	13

5.4	Key issue 4: Network slice fault management capability	13

5.5	Key issue 5: Communication service management exposure	14

5.6	Key issue 6: Application layer QoS verification capability enablement	14

5.7	Key issue 7: Network slice related performance and analytics exposure	15

5.8	Key issue 8: Support for requirements translation	15

5.9	Key issue 9: Support for trust enablement	16

5.10	Key Issue 10: Support for managing trusted third-party owned application(s)	17

5.11	Key issue 11: Dynamic slice SLA alignment	17

5.12	Key issue 12: Network slice capability exposure in the edge data network	18

5.13	Key issue 13: Delivery of the existing Network Slice information to the trusted third-party	18

5.14	Key issue 14: Network Slice creation to the third-party and UE	19

6	Solutions	20

6.0	Mapping of Solutions to Key Issues	20

6.1	Solution 1: Automatic application layer network slice management	20

6.1.1	Solution description	20

6.1.1.1	General	20

6.1.1.2	Automatic application layer network slice lifecycle management	21

6.1.2	Solution evaluation	22

6.2	Solution 2: Network slice fault management capability	22

6.2.1	Solution description	22

6.2.1.1	General	22

6.2.1.2	Network slice fault management capability exposure	23

6.2.2	Solution evaluation	24

6.3	Solution 3: Slice API configuration and translation	24

6.3.1	Solution description	24

6.3.1.1	General	24

6.3.1.2	slice API configuration	25

6.3.1.3	Slice API translation	26

6.3.2	Solution evaluation	27

6.4	Solution 4: QoS verification capability	28

6.4.1 Solution description	28

6.4.1.1	General	28

6.4.1.2	QoS verification capability	28

6.4.2	Solution evaluation	29

6.5	Solution 5: Network slice related performance and analytics exposure	29

6.5.1	Solution description	29

6.5.1.1	General	29

6.5.1.2	Network slice related performance and analytics exposure	29

6.5.1.2.1	KQI and performance data report from NSCE client	31

6.5.2  Solution evaluation	32

6.6	Solution 6: VAL server authorization and authentication via slice enabler layer	32

6.6.1	Solution description	32

6.6.1.1	General	32

6.6.1.2	VAL server authorization and authentication via slice enabler layer	33

6.6.2	Solution evaluation	33

6.7	Solution 7:  network slice capability registration	34

6.7.1	Solution description	34

6.7.1.1	 General	34

6.7.1.2 	Capability exposure registration	34

6.7.2	Solution evaluation	34

6.8	Solution 8: Discovery of management service exposure	35

6.8.1	Solution description	35

6.8.1.1	General	35

6.8.1.2	Procedure	35

6.8.2	Solution evaluation	37

6.9	Solution 9: Support for managing trusted third-party owned application(s)	37

6.9.1	Solution description	37

6.9.1.1	General	37

6.9.1.2	Network slice quota management capability exposure	37

6.9.2	Solution evaluation	38

6.10	Solution 10: Network slice application policy management capability	38

6.10.1	Solution description	38

6.10.1.1	General	38

6.10.1.2	Network slice application policy management capability	38

6.11	Solution 11: Communication service management exposure	40

6.11.1	Solution description	40

6.11.1.1	General	40

6.11.1.2	Use Case of Communication service lifecycle management	40

6.11.1.3	Communication service creation	43

6.11.1.4	Communication service modification	44

6.11.1.5	Communication service disengagement	44

6.X	Solution #X: <title>	44

6.X.1	Solution description	44

6.X.2	Solution evaluation	44

7	Overall evaluation	45

8	Conclusions	45

Annex A: Change history	46

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The study identifies the SA1 requirements and potential vertical requirements of network slice capability exposure that have yet to be realized, and proposes application architecture aspects solutions and enhancements to SEAL. Based on any gaps found between requirements and existing capabilities, the present document includes: identification of key issues, solutions, corresponding evaluations and conclusions to ensure the efficient network slice capability exposure.

The study takes into consideration the SEAL architecture specified in 3GPP TS 23.434 [2],  network slicing related architecture and services(such as services of NEF) specified in 3GPP TS 23.501 [3] and 3GPP TS 23.502 [4], network slicing management capability provisioning and architecture specified in 3GPP TS 28.531 [5] and 3GPP TS 28.533 [6], requirements specified in TS 22.261 [7].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[3]	3GPP TS 23.501: "System architecture for the 5G System (5GS)"

[4]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[5]		3GPP TS 28.531: "Management and orchestration; Provisioning".

[6]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[7]	3GPP TS 22.261: "Service requirements for the 5G system".

[8]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[9]	3GPP TR 22.835: "Study on enhanced access to and support of network slices".

[10]	3GPP TS 28.545: "Management and orchestration; Fault Supervision (FS) ".

[11]	3GPP TS 28.535: "Management and orchestration; Management services for communication service assurance; Requirements".

[12]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[13]	3GPP TS 28.550: "Management and orchestration; Performance assurance".

[14]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[15]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics".

[16]	3GPP TS 32.111-1: "Management and orchestration; Fault management, Part 1: 3G fault management requirements".

[17]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[18]	3GPP TS 22.263: "Services and System Aspects; Service requirements for video, imaging and audio for professional applications (VIAPA)".

[19]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[20]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[21]	3GPP TS 33.434: "Security aspects of Service Enabler Architecture Layer (SEAL) for verticals".

[22]				3GPP TS 29.536: "5G System; Network Slice Admission Control Service; Stage 3".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

Auto-NS-LCM	Automatic Application Layer Network Slice Lifecycle Management

ASP	Application Service Provider

EDN	Edge Data Network

KPI	Key Performance Indicator

KQI	Key Quality Indicator

NEF	Network Exposure Function

NEST	Network Slice Template

NOP	Network Operator

NSCE	Network Slice Capability Enablement

NsaaS	Network Slice as a Service

NSI	Network Slice Instance

NSSI	Network Slice Subnet Instance

NWDAF	Network Data Analytics Function

NSACF	Network Slice Admission Control Function

OAM	Operation, administration and maintenance

QoE	Quality of Experience

S-NSSAI	Single Network Slice Selection Assistance Information

VAL	Vertical Application Layer

# 4	Application architecture for network slice capability enablement

## 4.1	Architectural requirements

### 4.1.1	General requirements

[AR.4.1.1-a]	The application enablement layer shall support interaction with 3GPP network management system to consume network slice management service.

NOTE: The consuming of the network slice management service related procedures are specified in TS 28.531[5].

NOTE: The NSCE layer acts a service consumer utilizing the management services exposed by EGMF as defined in SA5 TS 28.533[6] if authorized.

[AR-4.1.1-b] The NSCE architecture shall support one or more applications from the same vertical.

[AR-4.1.1-c] The NSCE client shall be able to communicate to multiple NSCE servers.

### 4.1.2	Security requirements

[AR-4.1.2-a] The application architecture shall provide mechanisms to authorize the usage of network slicing related services by the VAL servers and NSCE clients.

[AR-4.1.2-b] The application architecture shall support mutual authentication and authorization check between clients and servers, servers and servers that interact.

NOTE: The authentication and authorization aspects related to VAL servers and NSCE enablers are out scope of this study and to be addressed by SA3, in TS 33.434[21].

### 4.1.3	Registration requirement

[AR.4.1.3-a] The application layer architecture shall provide mechanisms for an VAL server to register onto the NSCE servers.

### 4.1.4	Discovery requirement

[AR.4.1.4-a] The application layer architecture shall provide mechanisms for the discovery of NSCE services exposure to the VAL server.

### 4.1.X	<title of requirements#X>

Editor's Note:	This subclause will describe the application layer support aspect requirements.

## 4.2	Application architecture

### 4.2.1	General

This clause provides the overall architecture description:

-	Clause 4.2.2 describes the application architecture in the service based representation and reference point representation;

-	Clause 4.2.3 describes the functional entities;

### 4.2.2	Architecture

# Figure 4.2.2-1 Architecture for network slice capability enablement – Service based representation

Figure 4.2.2-1 exhibits the service-based interfaces for providing and consuming network slice capability enablement services.

The mechanisms for service discovery in the service-based representation depicted in figure 4.2.2-1 are as follows:

-	The network slice capability enablement server could provide service to VAL server and NSCE client through interface Snsce.

Figure 4.2.2-2 illustrates the service-based representation for utilization of the 5GS network services based on the 5GS SBA specified in 3GPP TS 23.501[3] and TS 28.533[6]

.

Figure 4.2.2-2 Architecture for network slice capability enablement utilizing the 5GS network services based on the 5GS SBA – Service based representation

Figure 4.2.2-3 depicts the network slice capability enablement architecture in the non-roaming case, using the reference point representation showing how various entities interact with each other.

Figure 4.2.2-3 Architecture for network slice capability enablement – reference points representation

The network slice capability enablement client communicates with the network slice capability enablement server over the NSCE-UU reference point. The network slice capability enablement client provides the support for network slice capability enablement functions to the VAL client(s) over NSCE-C reference point. The VAL server(s) communicates with the network slice capability enablement server over the NSCE-S reference point. The network slice capability enablement server, acting as AF, may communicate with the 5G Core Network functions via NEF (N33) reference point (for interactions with PCF, NSACF, etc.), or interacting with PCF directly via N5, if permitted. The network slice capability enablement server may interact with OAM system over NSCE-OAM reference point, as consumer in both NSaaS and NoP model (for Network Slice Provisioning capabilities, Performance Assurance,   Fault Supervision etc.).

NOTE:	The NSCE-OAM reference point can be realized by the services exposed by EGMF as defined in TS 28.533[6].

### 4.2.3	Functional elements

#### 4.2.3.1	Network slice Capability Enablement client

The network slice capability enablement client functional entity acts as the application client for the slice enablement. The network slice capability enablement client interacts with the network slice capability enablement server to trigger a network slice related operations. This trigger may be due to an application QoS requirement change, a service operation change, a network slice status change, etc. The NSCE client may receive a network slice related notification from the NSCE server. The NSCE client may optionally notify the VAL client on the network slice / DNN change.

#### 4.2.3.2	Network slice Capability Enablement server

The network slice capability enablement server functional entity provides application layer enablement to support the network slice management and control with/without invoking control and management plane capabilities from SA2 and SA5 pertaining to network slicing. Such enablement supports the network slice related operations such as the mapping or migration of one or more vertical applications to one or more network slices, triggering the dynamic network slice lifecycle management, NSI/NSSI monitoring, etc.

### 4.2.4	Service-based interfaces

The architecture for enabling network slice capability enablement service contains the following service-based interfaces:

Sval:	Service-based interface exhibited by VAL server.

Snsce:	Service-based interface exhibited by NSCE server.

Unsce-c:	Service-based interface exhibited by NSCE client.

NOTE:	Detailed specification of Unsce-c is based on implementation.

### 4.2.5	Reference points description

#### 4.2.5.1	VAL-UU

The interactions related to vertical application layer support functions between VAL client and VAL server are supported by VAL-UU reference point. This reference point is an instance of Uu reference point as described in 3GPP TS 23.401 [9] and 3GPP TS 23.501 [10].

NOTE:	The details of VAL-UU reference point is out of scope of the present document.

#### 4.2.5.2	NSCE-UU

The interactions between a NSCE client and the corresponding NSCE server are generically referred to as NSCE-UU reference point.  This reference point supports fault data, QoE data and KQI data provisioning etc.

#### 4.2.5.3	NSCE-C

The interactions between the VAL client(s) and the NSCE client(s) within a VAL UE are generically referred to as NSCE-C reference point.This reference point supports obtaining information about network slice that VAL client(s) require, application client information (such as its KQI) provisioning etc.

#### 4.2.5.4	NSCE-S

The interactions between the VAL server and the SEAL server are generically referred to as SEAL-S reference point.  This reference point supports network slice capability exposure such as: application layer slice lifecycle management, fault diagnosis, slice API configuration and mapping, QoS verification, slice performance analytics exposure etc.

# 5	Key issues

## 5.1	Key issue 1: Network slice capability management enhancements

SEAL is the service enabler architecture layer common to all vertical applications over 3GPP systems. It provides the functions like location management, group management, configuration management, identity management, key management, network resource management and network slice capability management as defined in 3GPP TS 23.434 [2].

Network slicing is a general network capability which can be applied for many vertical industries. The network slice capability management service in 3GPP TS 23.434 [2] only provides capability of network slice adaptation between NSCE server and the NSCE client. However, for such capability both control plane and management plane interactions need to be considered since there is close coupling between the per UE session and the per slice related actions. For example, the SEAL/NSCE layer may need to be aware of the slice provisioning parameters (NSI/NSSI configuration) which can be provided by the slice management system. Furthermore, NSI/NSSI performance monitoring from management system (e.g. NSI/ NSSI status) may be useful to be known at the enabler layer, since this may affect the application to slice re-mapping triggering (e.g. to re-map to the least congested slice); and may also impact other SEAL provided functionalities (e.g. QoS/resource control, group management etc).

Open issues:

-	Whether and which enhancement to SEAL network slice capability management service is required having in mind SA2 and SA5 slice related exposure?

-	Whether a potential enhancement to SEAL network slice capability management functional model is required?

-	Whether new or enhanced APIs are needed to support the potential SEAL enhancements?

-	How the network slice capability (such as management service (MnS)) consumption may trigger and impact the value-add services provided by the SEAL layer?

## 5.2	Key issue 2: Application layer exposed network slice lifecycle management

It is specified in clause 6.10 of 3GPP TS 22.261 [7] that 5G network is required to provide suitable APIs to allow a trusted third-party to create, modify, scale and delete network slices used for the third-party. As specified in 28.531 [5], SA5 has defined the network slice lifecycle management service. SA6 can provide a more concise application layer exposed network slice lifecycle management with additional functionality for verticals. It can help vertical do the lifecycle management without knowledge of SA5 O&M defined interfaces and APIs. In addition, with the network slice related information (such as network slice status reported from NSACF), the SA6 could trigger some slice lifecycle management operations automatically to provide a value-added lifecycle management service. It is not clear how to expose the network slice lifecycle management service from application perspective with providing additional value on top of existing SA5 solution.

Editor's note: The interface which may be used in this KI specified in TS 28.532 [8] is still under the discussion of SA5, and whether the enabler layer could use the interface directly or act as an entity to trigger the relevant procedures depends on SA5's decision.

Hence, it is required to study the following issues:

-	How to integrate and expose the SA5 and SA2 network slice related capability to provide some value-added lifecycle management service from application layer?

-	Whether and how additional service APIs are required to be supported for application layer enablement of network slice lifecycle management?

-	Whether and how CAPIF can be leveraged for additional service APIs.

## 5.3	Key issue 3: Discovery & registration aspects for management service exposure

There are use cases (being discussed also in SA1, TR 22.835 [9]), where the applications (e.g. gaming or online video applications) may access the 5GS over multiple slices for different services (e.g. based on the user membership); or have different priorities on different slices based on Application Service Provider (ASP) request. As an example, a mobile network operator has provisioned a set of network slices (Slice#1, Slice#2, Slice#3) which may be used by different ASPs (e.g. Slice#1 for online video services, Slice#2 for gaming.  Slice#3 for eMBB or IOT service). Different ASPs may use these slices (or a subset of them) for different services that they offer. Furthermore, when an application changes the network slices to be accessed, it should be agnostic to the UEs accessing the service and should be performed automatically.

The vertical enablement layer (SEAL, vertical-specific enablers) supports the exposure of telco provided services to the vertical / application service provider (ASP). Such telco-provided services traditionally covered the 3GPP control plane services (provided by SA2); however, these can be extended to 3GPP management domain services (provided by SA5) which may be useful for allowing the ASP to monitor and manage the slices used by the UEs. The enabler layer can be seen as a trusted application entity that interacts with the management system on behalf of ASP to allow the exposure of management services related to the offered slices. It can be used to trigger dynamic slice-related actions and also reduce complexity at the ASP side.

In this extended notion of vertical enablement (to cover the 3GPP management service exposure), the vertical/ slice customer needs to be able to 1) discover the relevant Network Slice Instances and the respective capabilities such as coverage offered, RAT/ frequencies, to 2) discover the management services (based on TS 28.533 [6]) which can be exposed as part of the offered slices, and 3) register to the 3GPP management domain via the vertical enabler layer for consuming the management services.

This study needs to investigate:

-	Whether and how SEAL needs to be enhanced to support the discovery of the offered slices and the offered management services related to these slices, to the vertical/ASP;

-	Whether and how SEAL needs to be enhanced to support the registration of the vertical applications to the 3GPP management domain.

## 5.4	Key issue 4: Network slice fault management capability

As the requirement captured in TS 22.261 [7], based on operator policy, the 5GS shall provide suitable APIs to allow a trusted third-party to monitor the network slice used for the third-party according to operator policy.

In addition, the descriptions of network diagnostics are captured in clause F.2, TS 22.261 [7] as following:

Network diagnostics helps with scanning, diagnosing and identifying problems within a network. Diagnostics includes gathering data and continuously providing sufficient performance parameters that characterize the quality of the network connection. This includes data of the physical connection as well as of logical links and sub-networks. Exposure of relevant (and possibly aggregated) performance parameters ensures a quick reaction in case of failure as well as identifying network connectivity, performance and other related problems.

Network diagnostic information needs to be generated automatically and, in case of a hosted or virtual network deployment, be made available to the tenant of the network via a suitable API.

The alarm data can be used to help the third-party to diagnose the fault problem of the services, locate the fault causes, and to be aware of the potential fault. In TS 28.545 [10], the fault supervision management services are standardized by which the alarm of the network slice instance from network resource aspects can be subscribed and reported. This alarm information together with the application function's fault report and communication service related knowledge can be utilized by the SEAL/NSCE to diagnose and locate the cause of the service performance deterioration and the fault of the communication services, and then exposed the fault report to the third-party. For example, if the status of the required communication is not correct, the SEAL/NSCE derives this alarm information from application functions. In this case, it is the SEAL/NSCE's responsibility to detect whether this fault is caused by the 5GS network or not and exposed the fault report to the third-party. If it is, then the SEAL/NSCE may inform the management functions the location of the fault and ask for the maintenance of the managed functions to clear the fault.

In addition, the two business relationships of network slice as a service and network slice as NOP internals may considered separately for the capabilities exposed in each scenarios. The coordination with SA5 is needed to fill the gap between the communication services (from the verticals' point of view) and the 5G network.

Open issues:

-	Whether and how additional APIs dedicated to network slice fault management capability are required?

-	How to define the APIs to expose the fault report to verticals client/server

-	How to coordinate with fault supervision management services provided by O&M systems defined in SA5 to fill the gap in fault management.

## 5.5	Key issue 5: Communication service management exposure

In clause 6.10 of TS 22.261 [7], following requirements are defined:

Based on operator policy, the 5G network shall expose a suitable API to allow an authorized third-party to define and reconfigure the properties of the communication services offered to the third-party.

The 5G system shall support the means for disengagement (tear down) of communication services by an authorized third-party

In addition, in clause 6.23.1 of TS 22.261 [7], it defines that QoS monitoring can be used for assessing and assuring the dependability of the communication services.

In SA5 TS 28.535 [11], management services to assure the communication service as per agreement (for example a SLS) with a communication service (only network slice as a service scenario) consumer (e.g. enterprise) have been defined. For other scenarios, the communication services from the verticals aspects required by SA1 (e.g., vertical automation communication services, URLLC services) are not discussed in SA5

From the vertical industry perspective, they may be more concerned about how to operate the communication services provided by 5GS to meet the business requirements on application level. Take V2X service as an example, there is a specific service of cooperative driving for vehicle platooning information exchange, and the vertical has some requirements on this service, including end-to-end latency between two UEs. Therefore, the service enable layer needs to provide such capabilities for vertical industries, including lifecycle management and quality assurance of communication services from the verticals' perspective. For example, with the QoE/QoS data of communication services collected from vertical application layer, SA6 may need to provide some value-added communication services management and/or assurance services.

It is not clear how to expose the communication service management service from application perspective with providing additional value on top of existing SA5 solution.

Hence, it is required to study the following issues:

-	What kinds of service APIs are required to be supported for application layer exposure of communication services life cycle management?

-	What kinds of additional service APIs are required to be supported for application layer exposure of communication services SLA assurance?

-	How to support above capabilities in the enable layer?

-	How to coordinate with SA5 functionalities to fill the gap between the communication services (from the verticals' point of view) and the 5G network.

## 5.6	Key issue 6: Application layer QoS verification capability enablement

In clause 6.23 of TS 22.261 [7], it is defined the requirements about QoS monitoring that 5G system shall be able to assessing and assuring the dependability of the communication services.

In clause F.1 of TS 22.261 [7], it is discussed how QoS monitoring information can be used for assurance purposes. In step "Customer rating of QoS", it mentions that the customer can compare the QoS achieved by the provider with the QoS requirements and its own experience of the QoS.

In clause SA5 TS 28.552 [12], it defined QoS measurements reports with different Filters from perspective of OAM, e.g. 5QI, QIC, S-NSSAI, PLMN.

In some cases, the verticals and the service provider reach an agreement on the SLA, however with this SLA requirement, the VAL client may also suffer unsatisfied experience. Hence, it could be possible to provide Vertical application layer the capability of comparing the QoS achievement status together with the OAM QoS data versus real customer QoS data (e.g., MOS) collected from VAL client to check whether the existing QoS data is able to satisfy the VAL client's.

It is not clear how to expose the QoS verification capabilities on top of existing SA5 solution.

Hence, it is required to study the following issues:

-	What kinds of additional service APIs are required to be supported for application layer enablement of QoS verification?

-	What kinds of additional service APIs are required to obtain the real vertical QoS to support the QoS verification?

-	How to support above capabilities in the enable layer?

## 5.7	Key issue 7: Network slice related performance and analytics exposure

As specified in clause 6.10 of 3GPP TS 22.261 [7], 5G network is requested to support a 3rd party to get the network status information of a private slice dedicated for the 3rd party. The 5G network collects various kinds of data, such as performance measurements in TS 28.552 [12] and analytics data as specified in 3GPP TS 28.104 [15] and the analytics data from NWDAF and NSACF exposed by NEF. However, to get the information efficiently, the verticals are supposed to know, which entity to interact with to get the desired information, how to extract useful information from data which is collected using different statistical methods from different entities, which is challenging for some verticals. The network slice capability exposure enabler layer can aggregate and process the data from different source, making the network information exposure more orderly and easier to read. For example, for the slice related performance and analytics come from multiple sources, the enabler layer could help to organize and aggregate the information. For some applications utilizing multiple S-NSSAI, the enabler layer could help to organize and aggregate the information so that it is exposed and displayed based on the application level rather than slices level.

Editor's note: Whether this SID is going to study the interface which is used to get the performance measurements and analytics is FFS.

Hence, it is required to study the following:

-	Whether and how information about available (SA2 and SA5) slice performance and analytics should be exposed to a third party with added value?

-	Whether and how available slice performance and analytics related information could be aggregated or processed to support an efficient information exposure?

-	Whether and how additional service APIs are required to be supported at SEAL for the network slice measurements and analytics exposure?

-	Whether and how CAPIF can be leveraged for additional service APIs?

## 5.8	Key issue 8: Support for requirements translation

Requirements for network slice from different verticals may vary from one to another, in terms of performance and capability requirements. For example, for the performance related requirements, the live video streaming cares more on bandwidth while V2X cares more on latency and jitters. For the capabilities related requirements, V2X may requests the positioning while future factory may request the self-control and management. To satisfy the requirements, vertical has to interact with several 5GS entities and understand the specific network parameters, such as the attributes in the service profile as defined in TS 28.541 [14]. Slice enabler layer could act as a mediator between the vertical customer and the 5GS to decompose and translate the requirements.

Besides, the verticals are more focused on the KQI or QoE of the applications and services. For example, in a scenario of future factory, the robots are used to enable the intelligent delivering, the third-parties have a requirement on the application latency between the robots and the robots control system, the slice enabler may decompose this application latency requirement to 5GS network latency requirement (e.g., the network latency requirements specified in TS 28.541[14] as packet transmission latency (millisecond) through the RAN, CN, and TN part of 5G network or the UL/DL packet delay measurement between UE and PSA UPF for a QoS Flow defined in TS 23.501[3]) and forward it to the 5GS systems. Another example is that to support the service of the imaging for professional applications as defined in TS 22.261[7], the imaging system latency of the application as defined in TS 22.263[x] is required which may not only depends on the 5GS network transmission latency but also will be influenced by the network throughput and network bandwidth. The NSCE are responsible for the translation from the vertical's KQI/QoE requirements to 5GS network requirements based on the knowledge of the industry profiles and 5G network (the definition of the KQI/OoE is service and application specific and is out the scope of this study).

However, to cope with the various requirements, it is needed to specify the operations and procedure on how to translate them more efficiently, such as in the unified manner with unified format/model/template. In such way, the slice enabler layer is capable of translating requirements from the vertical, to service consumption or service API invocation and configurations to the respective 5GS domains. The service could be provided from control plane, management plane and SA6 slice enabler layer itself, or the combinations of services above.

There are two main aspects in requirements translation: 1) one is how the vertical requirements could be collected, whether and how the template is needed? 2) The other is how to transfer the requirements into actions, whether and how API translation is needed?

This key issue aims to discuss how to configure and translate the requirements to service consumption and configuration in a way that the slice capability exposure is 1) agnostic to the underlying telecom infrastructure, 2) hides the complexity of telecom infrastructure, 3) doesn't impact/restrict the level of exposure to the vertical and 4) that is resilient to dynamic changes that may happen due to application portability or telco-provided API status changes.

Therefore, the open issues include:

-	Whether and how the vertical requirements could be collected at the slice enabler to allow the translation to network service consumption and configuration,

-	Whether and how API translation is needed to support the requirements translation.

## 5.9	Key issue 9: Support for trust enablement

A vertical application may use the slice enabler services (which can be seen as a trusted 3rd party to the MNO) to request management services as well as control plane services for a new slice on demand, based on an agreement between the vertical and the network slice provider.

However, the creation of a new slice will require a form of trust between the vertical/end application and the 5GS (management and control plane) for authorizing/authenticating the application request and enabling the vertical app to consume management / control services related to the requested slice.

The vertical application may not be trusted by the OAM or the 5GC, and is therefore not able to access management and control services. Also, there can be two way of accessing these services after authorization, via direct exposure or indirectly via the enabler server.

So, the key issue will study:

-	How to enable the authorization/authentication of the vertical application to consume telco-provided services (management and control plane), based on the vertical applications request.

- 	How to enable the authorization/authentication of the vertical application to consume telco-provided services (management and control plane) indirectly via the slice enabler layer, based on the vertical applications request.

## 5.10	Key Issue 10: Support for managing trusted third-party owned application(s)

As per 3GPP TS 22.261 [7], it is possible for trusted third-party to use a dedicated network slice for diverse use cases. Further, it provides following requirement to manage applications:

"Based on operator policy, a 5G network shall provide suitable APIs to allow a trusted third-party to manage this trusted third-party owned application(s) in the operator's Service Hosting Environment."

It is also possible for the third-party to offer its consumers different contract qualities level (e.g. gold, silver and bronze). In clause 5.7.1 of 3GPP TR 22.835 [9], following use case has been specified:

"For gaming or online video applications, the end users, who have subscription with MNOs who may provide multiple network slices to different users or services, may still have different priority or membership e.g. VIP maintained by 3rd party Service Provider (SP). And depending on the priority or membership information from 3rd party SP perspective, based on the agreement between SP and MNO, the UE have different priority for the available network slices."

In clause 4.2.11.2 of 3GPP TS 23.502 [4] specifies following:

"When for all the Requested S-NSSAI(s) provided in step 2 the NSACF returned the maximum number of UEs per network slice has been reached and if one or more subscribed S-NSSAIs are marked as default in the subscription data and not subject to Network Slice Admission Control, the AMF can decide to include these Default Subscribed S-NSSAIs in the Allowed NSSAI. Otherwise, the AMF rejects the UE request for registration. In the Registration Reject message the AMF includes the rejected S-NSSAI(s) in the rejected NSSAI parameter, and for each rejected S-NSSAI the AMF includes a reject cause to indicate that the maximum number of UEs per network slice has been reached and optionally a back-off timer."

Upon reaching maximum UEs slice quota, the 5GC may reject the registration request on the S-NSSAI from the gold quality level customer which may not be desirable by the trusted third party. This happens since 5GC is not aware of the relevant application information e.g. contract qualities level of the UE making the registration request.

The third party application needs to provide high priority to the higher level of contract qualities and so it needs to manage such connections.

It is also specified in 3GPP TS 22.261 [7] that the 5G system shall support a mechanism to optimize resources of network slices (e.g., due to operator deploying different frequency to offer different network slices) based on network slice usage patterns and policy (e.g., application preference) of a UE or group of UEs.

In particular, this KI will address:

-	Whether and how the AF can provide application policy information to the operator's Service Hosting Environment to enable automatic management of application resources?

-	What application policy information needs to be specified by the trusted third-party AF that can be automatically evaluated by the operator's Service Hosting Environment?

-	Whether and how the AF can manage use of the application resources in the operator's Service Hosting Environment on a per user characteristic?

## 5.11	Key issue 11: Dynamic slice SLA alignment

Some verticals/enterprises have strong desire for the slice/service self-management. Initially, they may order a slice with certain NEST (Network Slice Type) where the SLA parameters are already provided based on the static and estimated service requirements. After the service is executed on the slice, the slice may not fully match the service real-time running conditions, for example, maybe only 60% of the slice resource is used to support the service, and rest of the slice resource is always idle, or the slice resource is insufficient due to under-provisioning. In order to achieve the maximum return of investment and slice/service self-management, the verticals/enterprises expect a more optimal service profile which contains the exact value of those SLA parameters to support to the executing services' demands, i.e., dynamic SLA alignment.

However, the verticals/enterprises may not fully understand the slice SLA parameters, or they may not set proper SLA parameters initially. How to enable better alignment between the vertical needs and the SLA parameters needs to be studied.

NOTE:	All the potential changes to the slice are based on existing SA2 and SA5 service/capability.

Open issues:

-	Whether and How the SEAL network slice capability management service supports better alignment between vertical needs and SLA parameters for initial service requirements and ongoing service conditions?

## 5.12	Key issue 12: Network slice capability exposure in the edge data network

The network slice deployed in the core network has a certain distance from the customer, so the delay will be affected to a certain extent and cannot meet the operation requirements of low latency equipment. Moreover, a variety of service data need to be processed in the core network, the scale of data traffic is large, the backhaul network needs to bear a large load and consume more bandwidth. For example, the differential protection service has strict requirements for latency in power industry.

In order to meet the personalized services requirements of vertical industries, network slices are deployed by using the computing, storage and communication capabilities of the edge date network, so as to realize the localized processing of the service, reduce the service transmission latency and enhance the service performance.

How to expose the network slice capability deployed in the edge data network to the vertical industry or the third parties is worthy of our study.

Open issues:

-	How could the NSCE server deployed inside the EDN interact with the NSCE server outside the EDN?

-	Whether and how could the NSCE server inside the EDN manage the network slice that has resource outside the EDN?

-	Whether and how does SEAL need to be enhanced to support NSCE client to interact with NSCE server in the EDN and NSCE server outside the EDN?

## 5.13	Key issue 13: Delivery of the existing Network Slice information to the trusted third-party

There are many requirements on Network Slice Exposure specified in clause 6.10 of 3GPP TS 22.261 [7].

Based on operator policy, a 5G network shall provide suitable APIs to allow a trusted third-party to create, modify, and delete network slices used for the third-party.

Based on operator policy, the 5G network shall provide suitable APIs to allow a trusted third-party to monitor the network slice used for the third-party.

Based on operator policy, the 5G network shall provide suitable APIs to allow a trusted third-party to define and update the set of services and capabilities supported in a network slice used for the third-party.

Based on operator policy, the 5G network shall provide suitable APIs to allow a trusted third-party to configure the information which associates a UE to a network slice used for the third-party.

Based on operator policy, the 5G network shall provide suitable APIs to allow a trusted third-party to configure the information which associates a service to a network slice used for the third-party.

Based on operator policy, the 5G network shall provide suitable APIs to allow a trusted third-party to assign a UE to a network slice used for the third-party, to move a UE from one network slice used for the third-party to another network slice used for the third-party, and to remove a UE from a network slice used for the third-party based on subscription, UE capabilities, and services provided by the network slice.

In order to satisfy the above requirements, the third party should know the Network Slice existence in prior to the proper operation related to the Network Slice.

The 3rd party service providers may know the existence of Network Slice through a way of delivery from the NSCE as an entity of NSP, which manages information of Network Slice for the 3rd party service providers. The concept of Network slice delivery is described in TS 28.530 clause 4.1.8.

Which information should be contained and delivered is another important point for delivery of the existing Network Slice. When 3rd party service providers activate the existing Network slice, 3rd party service providers should analyze the specific information of Network Slice such as set of service, capability, QoE, bandwidth and so on in order to figure out to meet their service requirements. What kind of information is necessary to 3rd party service providers may vary according to the services of the 3rd party service providers. With the regards, NSCE servers collect and manage information on Network Slice which 5GS may provide. It may be enough to indicate the standardized NEST (e.g, MBB, URLLC, massive IOT, etc.). Security aspects have to be considered. If there is a need for resource reservation per customer, then a dedicated slice is needed.

Open issues:

-	When does the Network Slice delivery occur?

-	Which information of Network Slice does need to be contained when delivered? Or is it enough the delivery indicates the standardized NEST such MBB, URLLC, massive IOT?

-	Whether and how management domain capabilities (e.g. MnS) corresponding to the network slice need to be delivered via the NSCE server?

-	How the deployment of NSCE server (e.g. at NSP or NSC side) affects the level of slice information to be delivered?

## 5.14	Key issue 14: Network Slice creation to the third-party and UE

The requirements specified in clause 6.10 of 3GPP TS 22.261 [7] says,

Based on operator policy, a 5G network shall provide suitable APIs to allow a trusted third-party to create, modify, and delete network slices used for the third-party.

This requirement interprets that the third-party can allocate the Network Slice for its service with suitable APIs provided by 5G Network. It can be implemented based on whether an appropriate Network Slice for the third-party service does exist or not. In case the appropriate Network Slice exists, the third-party request to use the Network Slice for the service, and the corresponding entities modifies the info of the Network Slice properly. In case the appropriate Network Slice does not exist, then the third-party request to create the new Network slice. This procedure should be handled with interaction between VAL server, NSCE and 5G network. Security aspects have to be considered. If there is a need for resource reservation per customer, then dedicated slice is needed.

After the Network Slice allocation is requested, UE should be notified that the Network Slice for the third-party service is used. It is obvious that the UE utilizes the info of Network Slice in URSP or local configuration to transport the data of the third-party service. With the regard, without notification to the UE, the Network Slice will not be used for the service. When notified, SA6 needs to take it consideration whether the notification is transported over NSCE layer. If transported over NSCE layer, then it should be considered whether UE applies the new allocated Network Slice into URSP.

NOTE 1:	The application enablement layer will not circumvent nor try to replace solutions required at the network layer defined by SA2 nor the management layer defined by SA5.

Open Issues

-	What kind of information should be required for creating a new Network Slice for the third-party service (between NSP and NSC)? Any minimum set of information should be considered such as Service/Slice Type, Device Type or QoS Index?

-	Whether and how is the notification transported over NSCE layer?

-	Whether does UE apply the new allocated Network Slice into URSP when transported over NSCE layer?

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

# Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key issue 1 | Key issue 2 | Key issue 3 | Key issue 4 | Key issue 5 | Key issue 6 | Key issue 7 | Key issue 8 | Key issue 9 | Key issue 10 | Key issue 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solution 1 |  | X |  |  |  |  |  |  |  |  |  |
| Solution 2 |  |  |  | X |  |  |  |  |  |  |  |
| Solution 3 |  |  |  |  |  |  |  | X |  |  |  |
| Solution 4 |  |  |  |  |  | X |  |  |  |  |  |
| Solution 5 |  |  |  |  |  |  | X |  |  |  |  |
| Solution 6 |  |  |  |  |  |  |  |  | X |  |  |
| Solution 7 |  |  | X |  |  |  |  |  |  |  |  |
| Solution 8 |  |  | X |  |  |  |  |  |  |  |  |
| Solution 9 |  |  |  |  |  |  |  |  |  | X |  |
| Solution 10 |  |  |  |  |  |  |  |  |  | X |  |
| Solution 11 |  |  |  |  | X |  |  |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |

## 6.1	Solution 1: Automatic application layer network slice management

### 6.1.1	Solution description

#### 6.1.1.1	General

This solution aims to address the issues identified in Key Issue 2.

This solution provides a procedure for automatic application layer network slice management performed by the network slice capability enablement server based on network slice status collected from 5GS and QoE collected from application layer.

When network slice capability enablement server receives a request for automatic application layer network slice management from VAL server, the network slice capability enablement server performs the service operations including detecting and subscribing the event which may trigger the automatic network slice lifecycle management, making the network slice lifecycle management recommendation/decision, triggering the network slice management operations, notifying the consumer about the network slice information.

#### 6.1.1.2	Automatic application layer network slice lifecycle management

Figure 6.1.1.2-1 illustrates an automatic application layer network slice lifecycle management solution based on network slice related data and QoE collected from application layer.

Pre-conditions:

1.	The VAL client has requested a network slice provisioning;

2.	The VAL server has subscribed to the network slice capability enablement server for automatic network slice management;

3.	The network slice enabler layer is registered/capable for interacting with 5GS such as triggering network slice LCM operations, and has collected current network slice capabilities.

# Figure 6.1.1.2-1: Automatic application layer network slice lifecycle management

1.	The vertical server sends a request for automatic application layer network slice lifecycle management (auto-NS-LCM), with network slice requirements (e.g. delay, throughput, load, the maximum number of users supported, etc.). The request can indicate the level of auto-NS-LCM that is required, such as whether to notify the VAL server/consumer before performing the auto-NS-LCM. The request may also indicate the trigger conditions, such as by providing the monitored parameters and the corresponding thresholds.

2.	After receiving the request, the network slice capability enablement server checks that the user is authenticated and authorized to perform the corresponding auto-NS-LCM operations, and filters the unauthorized requests, if any.

3.	If authenticated and authorized, the network slice capability enablement server, acting as the network slice LCM service consumer, triggers the AllocateNsi request (see TS 28.531 clause 6.5.1 [5]) towards the respective management service provider, based on the network slice capabilities and network slice requirements.

4.	The network slice capability enablement server sends the auto-NS-LCM response to the VAL server with the result(s) of the network slice LCM operation(s).

5.	According to network slice requirements, network slice capability enablement server triggers the provision of network slice status and QoE metrics.

5a.	The network slice status could be collected through subscribing or requesting to 5GS. For example, to monitor the slice load, it could subscribe to/request the relevant service(s), such as AnalyticsExposure defined in TS 23.502 [4] clause 5.2.6.16, provisioning data report exposure for NSI in clause 5 of TS 28.532 [8]. If the trigger conditions are not indicated in the subscription, the network slice capability enablement server may help to configure an appropriate trigger condition, such as report period or thresholds.

5b.	Also, the network slice capability enablement server could get the information of QoE metrics from the application layer domain.

6.	Once the trigger condition or a combination of trigger conditions are met, based on requirements and network slice capabilities with updated information in Step 5, such as change in QoE, change in network slice status etc., the network slice capability enablement server determines whether and what network slice LCM operations should be taken and makes the decision(s)/recommendation(s), such as modifyNsi/AllocateNsi/DeallocateNsi request as specified in TS 28.531 [5].

7.	Optionally, if it is indicated in the request to notify the VAL server/consumer before performing the auto-NS-LCM, the network slice capability enablement server sends the network slice LCM recommendation(s) with network slice status to VAL server, to see whether takes the recommendation(s) or not.

8.	Optionally, sending the response indicating the decision made by VAL server to network slice capability enablement server.

9.	Based on decision made by VAL server or network slice capability enablement server, the network slice capability enablement server performs the corresponding operation(s).

10. According to the corresponding operation(s) result, the network slice capability enablement server sends the response to the VAL server.

### 6.1.2	Solution evaluation

The proposed solution addresses Key Issue #2. SA5 provides the network slice lifecycle management, while this solution provides a procedure for automatic application layer network slice management performed by the network slice capability exposure server based on network slice status collected from 5GS and QoE collected from application layer. With automatic application layer network slice management, the consumer's requirement could be better met without having to interact with 5GS frequently. This solution does not introduce impact on 5GS architecture. It will possible need enhancement of SA6 SEAL architecture enhancement to support the interaction with OAM system, but the interface for interaction with the OAM system is still being studied in SA5.

## 6.2	Solution 2: Network slice fault management capability

### 6.2.1	Solution description

#### 6.2.1.1	General

This solution addresses the key issue 4 of network slice fault management described in clause 5.4.

This solution provides a possible procedure to illustrate the network slice fault management capability exposed by NSCE server.

To enable the VAL server to monitor the problems of a network, the NSCE layer should help the VAL server to scanning, diagnosing and identifying problems within a network. The performance data and alarm data from multiple sources is helpful to characterize the quality of the network connection.

#### 6.2.1.2	Network slice fault management capability exposure

Figure 6.2.1.2-1 illustrates the network slice fault management process to address the key issue 4 of network slice fault management described in clause 5.4.

Pre-conditions:

1.	The VAL client has subscribed to the network slice fault management capability.

2.	The network slice enabler layer is capable to interact with NEF and OAM system.

3.	The VAL client has checked the status of application layer.

# Figure 6.2.1.2-1 Network slice fault management process

1.	The VAL server sends a request to NSCE server to request for the fault diagnosis of the applications and networks when it detected a fault in the application layer, the services and the network slices. This request may be triggered by the errors of the applications detected by the VAL server itself, or the VAL server may periodically collect the fault diagnostic report subscribing to the network slice fault management capability. The fault information of applications may be included in this request. For example, the Service Availability Failure Events e.g, described as Fallback events carried out by the Dual SIM cellular device.

2.	On receiving the request from VAL server, NSCE server retrieves the performance data of network slice from 5GS. For OAM system, the APIs defined in clause 11.3, TS 28.532[8] is utilized. For CN functions, the APIs of Nnwdaf_AnalyticsInfo service defined in clause 7.3, TS 23.288[17] can be utilized.

The analytics data defined in clause 6.3 to clause 6.14, TS 23.288[17], network slice instance related performance data defined in clause 5, TS 28.552[12] and network analytics data in clause 8.3 and clause 8.4 TS 28.104[15] exposed by OAM system may be acquired.

3.	NSCE server retrieves the alarms of network slice instances from OAM system via the procedures defined in clause 6.1,TS 28.545[10], and the alarms are defined in clause 4.1.1.1, TS 32.111-1 [16], e.g., the fault of communication, environmental, equipment, processing error, QoS for device/resource/file/functionality/smallest.

4.	NSCE server may request the alarm information (e.g., the 5GS network is not work or the required performance is under the threshold which leads the service's problem) collected by NSCE client if possible. The information collected by the NSCE client depends on the third-parties' requirements and implementation.

5.	NSCE client report the requested fault information to NSCE server.

6.	NSCE server correlates the applications, services and the network resources of the specific network slice instances which supports the applications and services with the S-NSSAI, NSI ID (nSInstanceId defined in TS 28.541[14]), Application Identifier (defined in 23.501[3]), and then diagnoses the causes of the fault of the applications or services by analysing the fault information from different sources. For example, the RAN function of the slice instance which is utilized to support the service of the smart grid application, for a certain time duration, the smart grid suffered the bad experience caused by Service Availability Failure Events, and in the RAN function is detected continuously to report an alarm of environmental fault in the same time duration, then the environment fault may be the root cause of the Service Availability Failure and should be prioritized to be solved. The fault may be identified with "critical", "major", "minor", "ignore" to show its prioritization.

7.	NSCE server response to VAL server with the reports of fault diagnostic result to indicate the causes of the failures of the applications or services. If there is no problems within the 5GS network, the VAL server may be requested to check the application layer itself again.

8.	If the NSCE server detects that the application/service error is caused by the 5GS, the VAL server may send the fault diagnosis report to OAM system to indicate the server fault which causes the application/service failure by utilizing the NSCE-OAM interface.

NOTE: The APIs utilized to send the fault diagnosis report to OAM will re-utilize the fault management services in TS 28.532[8] exposed by EGMF as defined in TS 28.533[6].

### 6.2.2	Solution evaluation

The solution addresses the key issue #4 and this solution will have interaction with both the 5GC and OAM to collect the alarm information and network analytics data and also could acquire application fault data from VAL server or VAL client. For the NSCE server and NSCE client, it will possible need enhancement of the existed Rel-17 architecture to perform network and service level diagnosis. Also, fault diagnosis service in service level should also need to be added in the existed network architecture.

## 6.3	Solution 3: Slice API configuration and translation

### 6.3.1	Solution description

#### 6.3.1.1	General

This solution aims to address the issues identified in Key Issue 8.

This contribution provides a solution for the API configuration and translation and proposes the translation of the service API as invoked by the end applications to slice APIs based on the API configuration and application to slice mapping. Slice APIs can be defined as customized/tailored sets of service APIs (which can be either NEF northbound APIs or OAM provided APIs or enabler layer/SEAL provided APIs) and can be mapped to particular slice instances. The slice APIs can be a bundled or combined API comprising of different types of APIs, which will be used to expose the telco (5GS/SEAL)- provided services as needed by the applications of the slice customer. Each slice API may be configured per network slice instance.

This solution provides a support functionality to the vertical application specific layer and configures the exposure of APIs in a slice-tailored manner. It is assumed that the VAL server is not initially aware of the all the API exposure capabilities and information which will be needed for the given slice based on the SLA, and NSCE plays a vital role in configuring and translating the slice API based on the per slice requirements to service APIs.

This solution has two procedures:

-	a procedure on the slice API configuration

-	a procedure on the slice API translation

#### 6.3.1.2	slice API configuration

In this procedure, the VAL server initially provides an application requirement to enabler server including the service KPIs and the subscribed/preferred slices. Then, the slice enabler configures the mapping of the VAL application to a slice API which is a combination/bundling of northbound APIs (from both management and control plane). In particular, a slice API (or SDK) consists of telco-provided/platform dependent service APIs (e.g. NEF, OAM, SEAl, etc), and provides an abstraction/simplification on top of them. The procedure also covers the scenario where a trigger event occurs (e.g. QoS degradation, slice load) and the mapping configuration or the slice API configuration needs to change. In this scenario, the slice enabler updates the configuration of the API and provides a notification to the VAL server.

Figure 6.3.1.2-1 illustrates a solution for the slice API configuration.

Pre-conditions:

1.	The VAL server has registered to receive network slice capability enablement services

# Figure 6.3.1.2-1: slice API translation

1.	The VAL server sends a VAL application requirement request to the network slice capability enablement server. This request provides the service requirements / KPIs, the capability exposure requirements and a preferred/subscribed slice identification (e.g. S-NSSAI or ENSI)

2.	The network slice capability enablement server maps the VAL application requirement to a slice API which includes a list of APIs which are needed to be consumed as part of this service capability exposure (such APIs can be NEF APIs as specified in TS 29.522 clause 5 e.g. related to network monitoring, slice status, analytics exposure, SEAL APIs as specified in TS 23.434, OAM provided APIs). Such mapping can be determined at the network slice capability enablement server based on the VAL application exposure requirements or can be pre-configured per slice instance. The criteria for the mapping are the capability exposure requirement per slice (based on GST parameters, or from service/slice profile] as well as the capability exposure permissions/authorization for the API invoker.

The network slice capability enablement server may also store the mapping of the slice API to the service API list and per service API information (e.g. data encoding, transport technology, API protocol and versions)

Editor's Note: The exposure of MnS via OAM-provided APIs requires coordination with SA5 and is FFS

3.	The network slice capability enablement server subscribes/registers to consume the corresponding APIs from the 5GS (NEF and OAM) and SEAL service producers. For example, network slice capability enablement server may subscribe to consume NEF monitoring events, or SLA monitoring from OAM.

4.	The network slice capability enablement server sends a VAL application requirement response to notify on the result of the request and indicate whether configuration of the slice API can be possible or not.

5.	The network slice capability enablement server sends the slice API information and optionally the slice to service API mapping to the VAL server.

6.	A trigger event is captured by the service/API providers (e.g. control plane, management plane, SEAL) or the VAL server side (application server relocation to different EDN/DN, UE mobility to different EDN, application change of behavior) or any other API related event captured by the network slice capability enablement server (e.g. failure, unavailability, high load).

7.	The network slice capability enablement server processes the trigger event and checks and updates the mapping of service APIs to the slice APIs. The objective is to keep the slice APIs unchanged, so the VAL server is not aware of any change (if not triggered by VAL server). To accomplish this, the service APIs may need to be updated accordingly. For example, if one service API changes (e.g. due to high load, unavailability,  ..) the mapping to service APIs should be updated (e.g. if service API is a Location API which is provided by 5GC in the first place, the update would trigger the remapping to a Location API from SEAL LMS) to avoid affecting the slice API.

8.	The network slice capability enablement server updates the subscription/registration to the underlying 5GS and SEAL service producers, if an update on the service APIs (e.g. NEF APIs, SEAL APIs, OAM provided APIs) is needed.

9.	The network slice capability enablement server optionally notifies the VAL server on the slice/service API related updates.

#### 6.3.1.3	Slice API translation

This procedure follows the 6.3.1.2 and aims to describe how the slice API invocation request is translated to service API invocations after the slice API configuration mapping. In this procedure, the network slice capability enablement server initially receives a slice API invocation request from the vertical application based on step 5 of 6.3.1.2. Then, the network slice capability enablement server based on the slice API request fetches the service APIs to be invoked based on the slice API configuration and performs invocation requests to the corresponding service API providers.

As example, a slice API is requested for an IIOT slice. This may translate to: NSI Monitoring from Management Domain #1, NSSI Monitoring from Management Domain #2, network/QoS monitoring from NEF#1, Location monitoring from SEAL LMS, slice-related analytics from NWDAF (via NEF). Such translation could be based on the slice API configuration mapping of 6.3.1.2.

Figure 6.3.1.3-1 illustrates a solution for the slice API translation based on the configuration.

Pre-conditions:

1.	The VAL server has registered to receive network slice capability enablement services.

2.	The slice API mapping to the VAL server has been performed based on 6.3.1.2 step 2 and the slice API information is provided to the VAL server based on 6.3.1.2 step 5.

# Figure 6.3.1.3-1: slice API translation

1.	The VAL server sends a slice API invocation request to the network slice capability enablement server. This request provides slice API information (name, type, communication methods, protocols,..),  based on the received information in step 5 of 6.3.1.2. Service APIs are indicated above can be NEF APIs as specified in TS 29.522, SEAL APIs as specified in TS 23.434, OAM provided APIs)

Editor's Note: The exposure of MnS via OAM-provided APIs requires coordination with SA5 and is FFS

2.	After receiving the request, the network slice capability enablement server checks that the user is authenticated and authorized to perform the slice API invocation and maps the requested slice API to a service APIs (based on the VAL server ID / slice ID and procedure in clause 6.6.1.2).

NOTE 1:	If CAPIF is used, the network slice capability enablement server acts as AEF, and the authorization is obtained by CCF.

3.	The network slice capability enablement server generates a trigger for service API invocation requests to all the service APIs within the slice API.

4.	The network slice capability enablement server sends a service API invocation request to the producers of the service APIs within the slice API. This may include the originated service API invoker identity information (VAL server) as well as the direct API invoker information, authorization information, service API and slice API identification.

5.	The service API provider authenticates the API invoker and authorizes the request.

NOTE 2:	If CAPIF is used, the requests are sent to the corresponding AEFs of the API provider's domain, and the authorization is obtained by CCF.

6.	The network slice capability enablement server receives the service API invocation response as a result of the service API invocation.

7.	The network slice capability enablement server sends to the VAL server a slice API invocation response as a result of the slice API invocation.

### 6.3.2	Solution evaluation

Editor's Note:	This subclause will evaluate the solution.

## 6.4	Solution 4: QoS verification capability

### 6.4.1 Solution description

#### 6.4.1.1	General

This solution aims to address the issues identified in Key Issue 6.

This solution provides a procedure to illustrate the QoS verification capability by the network slice capability exposure server.

To enable the VAL server to verify QoS data of a network, the NSCE layer should help the VAL server to check whether the existing QoS data is able to satisfy the VAL client's requirement.

#### 6.4.1.2	QoS verification capability

Figure 6.4.1.2-1 illustrates the QoS verification capability process to address the key issue 6 of Application layer QoS verification capability exposure described in clause 5.6.

Pre-conditions:

1.	The VAL client has subscribed to the QoS verification capability.

2.	The VAL client has subscribed to the capability of Network slice related performance and analytics exposure.

3.	The network slice enabler layer is capable to interact with NEF/NWDAF and OAM system.

Figure 6.4.1.2-1 QoS verification process

1.	The VAL server sends a request to NSCE server to verify the service/application QoS data.

2.	On receiving the request form VAL server, NSCE server retrieves the QoS data of network slice from 5GC and OAM system. For OAM system, the APIs defined in clause 11.3, TS 28.532[15] is utilized, the performance data specified in TS 28.552[12] and analytics data specified in TS 28.104[16] can be collected. For CN functions, the APIs of Nnwdaf_AnalyticsInfo service defined in clause 7.3, TS 23.288[18] is utilized. The data of QoS information defined in TS 23.501[3].

3.	NSCE server sends a request to application layer to request the QoE data (e.g., MOS).

4.	VAL server sends the requested QoE data to NSCE server.

5.	NSCE server may send a request to NSCE client to collect the QoE data of network and services if possible.

6.	NSCE client report the requested data collected by itself if possible, the information collected by the NSCE client depends on the third-parties' implementations.

7. 	NSCE server verifies and analyses QoS data of network slice instance, Comparing QoS achieved status together with the OAM QoS data versus real customer QoE data (e.g., MOS) collected from VAL client to check whether the existing QoS data is able to satisfy the VAL client's to generate the QoS verification report as required by VAL server for specific period of time.

8. 	NSCE server sends the service/application QoS verification response to VAL server.

### 6.4.2	Solution evaluation

The proposed solution addresses Key Issue #6, the solution provides a procedure for application layer the capability of QoS verification by the network slice capability exposure server, which has interaction with both the 5GC and OAM to collect the QoS data from N33 and NSCE-OAM interface respectively, acquires network and service QoE data from NSCE client by NSCE-UU interface and obtains application QoE data from VAL server by NSCE-S interface to compare QoE date and QoS data, checking whether the existing QoS data is able to satisfy the real customer QoE data.

This solution does not introduce impact on 5GS architecture and SEAL architecture, the NSCE server has the capability to translate and compare QoE data and QoS data."

## 6.5	Solution 5: Network slice related performance and analytics exposure

### 6.5.1	Solution description

#### 6.5.1.1	General

This solution addresses the key issue 7 of performance and analytics exposure described in clause 5.7.

This solution provides a possible procedure to illustrate the network slice related performance and analytics exposure capability provided by network slice capability exposure server.

To enable the third-party to adjust the network slice or to configure the information which associates a service/UE to a network slice, the third-party is allowed to monitor the performance data and analytics data of the network slice, service or application. The network slice related performance data may be collected from multiple sources (e.g., OAM system or 5GC functions) to help the NSCE layer to perform data aggregation or analyses to generate the data required by the third-party. The solutions of performance data and analytics data exposed to the third-party needs to be studied.

#### 6.5.1.2	Network slice related performance and analytics exposure

Figure 6.5.1.2-1 illustrates the network slice related performance and analytics exposure process to address the key issue 7 of performance and analytics exposure described in clause 5.7.

Pre-conditions:

1.	The VAL client has subscribed to the capability of Network slice related performance and analytics exposure.

2.	The network slice enabler layer is capable to interact with NEF and OAM system.

# Figure 6.5.1.2-1 Network slice related performance and analytics exposure process

1.	The VAL server sends a request to NSCE server to collect the desired service/application specific performance data and analytics data, the detailed content of the reported data depends on the type of the application and services (the KQI and QoE data may be included with in the request). For example, the VAL may request the NSCE server to report the performance and analytics report of the service of the imaging for professional applications. The request may contain the interested geographical area.

NOTE 1:	It is not shown in the figure that VAL client could also trigger step1. It may be triggered by a specific event or request from the VAL client. Besides, the VAL client can trigger this procedure if the performance issue are detected by the VAL client itself.

NOTE 2:	Both periodical or ad-hoc reporting are possible, the VAL layer can request a periodical reporting or a one-off reporting. The result of the reporting can be subscribed by the VAL layer; it can request both or any one of them.

2.	The NSCE server shall check if the VAL server is authorized to get the network slice performance and analytics data.

3.	On receiving the request from VAL server, NSCE server retrieves the performance data and analytics data of network slice from 5GS. For OAM system, the APIs defined in clause 11.3, TS 28.532[8] is utilized. For CN functions, the APIs of Nnwdaf_AnalyticsInfo service defined in clause 7.3, TS 23.288[17] is utilized.

The performance and analytics data such as the Observed Service experience statistics, Load level information of a Network Slice defined in TS 23.288[18], network slice status defined in TS 23.502[4] clause 5.2.21, and analytics data defined in TS 23.288[17], network slice instance related performance data defined in TS 28.552[12] and network analytics data in TS 28.104[15] exposed by OAM system may be acquired. For example, when the performance and analytics report of the service of the imaging for a professional application is required, the imaging system latency as defined in TS 22.263[18] should be reported and analysed. In this case, the NSCE requests the OAM system to report the one-way/Round-trip packet delay of a specific S-NSSAI between PSA UPF and NG-RAN in clause 5.4, TS 28.552[12] and requests the NWDAF to report the User Data Congestion Analytics report defined in clause 6.8, TS 23.288[17].

4.	NSCE server retrieves the KQI data of services (e.g., the jitter duration of a video service), the network performance related data and the end users information (e.g., 5G UE's running time) by following the procedures defined in Figure 6.5.1.2.1-1 and Figure 6.5.1.2.1-2.

5.	NSCE server correlates and analyses the performance data of network slice instance, the analytics data of group of UEs and the KQI/QoE data to generate the performance data and analytics data report as required by VAL server for specific period of time.

NOTE 3:	How the data analytics correlation happens is out of scope of NSCALE study

6.  NSCE server sends the service/application related performance data and analytics response to VAL server. For example, when the performance and analytics report of the service of the imaging for a professional application is required, the report may acquire the calculated imaging system latency of the application, and level (e.g., good, normal, poor) of the latency.

##### 6.5.1.2.1	KQI and performance data report from NSCE client

Figure 6.5.1.2.1-1 and 6.5.1.2.1-2illustrate the procedure to acquire instant KQI/performance data and KQI/performance data report from NSCE client respectively.

# Figure 6.5.1.2.1-1 Process of instant KQI/performance data reporting

1.	NSCE server sends a request to NSCE client to ask for instantly reporting of the KQI and performance (e.g., the jitter duration of a video service), the network performance related data (e.g., the downlink/uplink RSRP of serving cell measured by 5G UE) and the end users information (e.g., 5G UE's running time).

Editor's Note: The APIs utilized to achieve the instant KQI data from NSCE client is to be defined in SA6.

2.	On receiving the request described in step1, the NSCE client sends the response to notify NSCE server whether the request is received successfully.

3.	NSCE client reports the requested data (e.g., the jitter duration of a video service, 5G UE's running time) once the data is measured.

NOTE 1:	In this study, general functionalities and procedures of NSCE are defined, the required data depends on the concrete scenarios. The data measured by NSCE client may depend on the study of SEALDD.

# Figure 6.5.1.2.1-2 Process of reporting instant KQI/performance data

1.	NSCE server sends a request to NSCE client to request the report collection of KQI data (e.g., the jitter duration of a video service), the network performance related data and the end users information (e.g., 5G UE's running time), and the time period of the data collection may be included in this request.

Editor's Note: The APIs utilized to achieve the fault data from NSCE client is to be defined in SA6.

2.	On receiving the request described in step1, the NSCE client sends the response to notify NSCE server whether the request is received successfully.

3.	NSCE server sends a request to retrieve the data desired report, the data type and the time duration may be indicated in this request.

4.	NSCE client sends the requested data report (e.g., the report of the jitter duration of a video service, 5G UE's running time) collected by itself.

NOTE 1:	In this study, general functionalities and procedures of NSCE are defined, the required data depends on the concrete scenarios. The data measured by NSCE client may depend on the study of SEALDD.

### 6.5.2 	Solution evaluation

The solution addresses the key issue #7 and this solution will have interaction with 5GC and OAM system to acquire KQI and performance data from N33 and NSCE-OAM interface, it also acquires KQI and performance data from NSCE Client by NSCE-UU interface. For the NSCE Client, it has the capabilities of KQI and performance data measurement and collection from Unsce-c interface. The data measurement may also depend with SEALDD. The NSCE Server could capture the KQI and performance data from 5GC and OAM system as well as NSCE Clients. The third applications or verticals can extract the performance data of network and analysis result from NSCE Server.

## 6.6	Solution 6: VAL server authorization and authentication via slice enabler layer

### 6.6.1	Solution description

#### 6.6.1.1	General

This solution aims to address the issues identified in Key Issue 9.

When a vertical application from third party wants to request management services as well as control plane services for a new slice on demand, it needs the authorization and authentication from 5GC. The vertical application may not be trusted by the OAM or the 5GC, it needs to interact with OAM and 5GC via the enabler server.

#### 6.6.1.2	VAL server authorization and authentication via slice enabler layer

Figure 6.6.1.2-1 illustrates the VAL server authorization and authentication via slice enabler layer.

Pre-conditions:

1.	The NSCE server can be seen as a trusted AF to interact with 5GC and has registered to the OAM.

2.	The NSCE server has been authorized to act as a middle layer to do the authentication and authorization.

# Figure 6.6.1.2-1 VAL server authorization and authentication via slice enabler layer

1.	The vertical server sends a request for authorization and authentication to the NSCE server with its identity and the requested network slice management and control capability (e.g. request for establish a new network slice).

2.	After receiving the request, the NSCE server authenticates and authorizes the VAL server based on its identity and requested capability.

The procedure of authentication and authorization could refer to API invoker authenticate and authorize as specified in TS 23.222[20] clause 8.1.14, 8.1.15 and 8.1.16, if applicable.

NOTE: the authentication and authorization mechanism will be in scope of SA3.

3.	Upon authorized and authenticated, NSCE server sends the authorization and authentication response and information to VAL server with the token.

4.	The NSCE server triggers the network slice management and control operation.

### 6.6.2	Solution evaluation

Editor's Note:	This subclause will evaluate the solution.

## 6.7	Solution 7:  network slice capability registration

### 6.7.1	Solution description

#### 6.7.1.1	 General

This solution aims to address the issues identified in Key Issue 3.

This solution provides a possible procedure to perform the VAL server registration.

The vertical enablement layer (SEAL, vertical-specific enablers) supports the exposure of telco provided services to the VAL. After registered in the enablement layer, the VAL server could be authorized to access network slice related exposure capabilities, e.g. monitoring of network/UE performance.

#### 6.7.1.2 	Capability exposure registration

# Figure 6.7.1.2-1 Capability exposure registration

1.	The VAL server sends the registration request to the NSCE server with its identity, and may also include interested service and level of exposure requirement.

2.	The NSCE server does the registration check by verifying whether all the necessary information has been provided and is reasonable. If the VAL server is already registered with the same registration information, the request will be rejected by the NSCE server. If the VAL server is already registered and updated information is received in the registration request, the request will be accepted by the NSCE server and the existing registration context of that VAL server will be updated based on the latest registration request.

The procedure of registration check could refer to API invoker on-boarding as specified in TS 23.222[20] clause 8.1.3 if applicable.

3.	After the registration procedure, the NSCE server sends the response to the VAL server indicating could initiate the authorization and authentication and further to do the service discovery.

### 6.7.2	Solution evaluation

Editor's Note:	This subclause will evaluate the solution.

## 6.8	Solution 8: Discovery of management service exposure

### 6.8.1	Solution description

#### 6.8.1.1	General

This solution aims to address the issues identified in Key Issue 3 and more specifically the first open issue supporting the discovery of the offered slices and the offered management services related to these slices, to the vertical/ASP.

To be able to utilise capabilities/features of the management system the applications must be made aware of existence of such features and capabilities. All management domain (MD) features/capabilities come with a pre-configured exposure, where this can be configured by the operator for a given slice. This exposure is used to decide which application can see which information regarding the capability/feature. Exposure refers to the permissions that the 3rd party entity has gained over its use of the management service, e.g., the ability to read, or execute or modify or delete can be considered as different sorts of exposure.

A MD feature/capability is anything of use offered by the management system to the 3rd party application. Therefore, a new feature could be a managed entity, a MnS or management API, any software, hardware, or other functionality – for example, new technology support, new coverage area, new network slice type or instance, new NFs or new network slice subnet type or instance.

This solution provides a mechanism which is aligned with SA5 ongoing work related to the MnS registry exposure for external discovery (see TR 28.824 clauses 5.4 and 7.1), which enables the initial discovery of MnS for a given slice based on VAL server request, and the discovery of new/modified MnS.

#### 6.8.1.2	Procedure

In this procedure, the VAL server initially requests the new MnS which are supported by a target slice and based on this request the NSCE server requests from the OAM the MnS discovery (optionally via EGMF). Then, the OAM/MnS registry derives the details to be exposed based on the NSCE server/VAL server permissions and provides the list of MnS for the given slice and the access details via the NSCE server to the VAL server. This procedure also includes the case when a new/modified MnS is deployed at the MD for the given slice, and the OAM/MnS registry provides this information directly to VAL server (assuming that VAL server has registered to the MD).

Editor's Note: Whether the interaction between NSCE server and MnS registry is direct or via EGMF is being discussed in SA5 and is FFS

Pre-conditions:

1.	The VAL server has registered to receive NSCE services

2.	The NSCE server is trusted by the 3GPP MD.

3.	MnS registry at OAM is aware of the allowed MnS and the exposure levels for a given slice

Figure 6.8.1.2-1 illustrates a solution for the MnS discovery support.

# Figure 6.8.1.2-1: MnS discovery support

The steps of this embodiment are as follows:

1.	The VAL server requests from NSCE server the expected exposure capability type and the related exposure level for a target slice or for a given VAL application, using the VAL application ID, the MD ID and optionally the slice ID.

2.	The NSCE server translates the expected exposure capability type in to MnS exposure requirement.

3.	The NSCE server coordinates with the 5GS and discover the related network service(s), by triggering the MnS service discovery as specified in TS 28.533[6], clause5.1.1 and 6.1.1. The 5GS, acting as the discovery service producer, sends the discovery result including related exposure information such as management service identifier, management service information and management service producer information to the NSCE server.

4.	The NSCE server sends the service/management data based on step 3, including the list of MnS and the corresponding exposure details to the VAL server. Here to mention that based on exposure level, the exposure details may be different.

5.	VAL server registers one or more VAL applications to MD for the management domain capability exposure for the subscribed slice.

Editor's Note: The registration procedure is FFS.

6.	The MnS registry discovers new or modified MnS in the 3GPP MD. The MnS registry derives the details to be exposed to the VAL server (via the NSCE server), based on the exposure level configured by the new or modified MnS and the exposure levels of registered VAL applications and NSCE server. The MnS change could be triggered due to changes on:

- New/modified management service producers

- New/modified managed entities – such as new radio, new technology or new NFs

- New/modified technical support – such as support in a new geography or coverage area

- availability of new management data – e.g. related to slice performance

7.	The MnS registry sends to the NSCE server the new / modified MnS, as discovered in step 6.

8.	The information on the discovered new/modified MnS is sent from the NSCE server to VAL server.

### 6.8.2	Solution evaluation

Editor's Note:	This subclause will evaluate the solution.

## 6.9	Solution 9: Support for managing trusted third-party owned application(s)

### 6.9.1	Solution description

#### 6.9.1.1	General

This solution addresses the key issue 10 for managing trusted third-party owned application(s) as specified in clause 5.10.

This solution provides a possible procedure to illustrate the process of the VAL server requesting to manage network slice quota to NSCE server. This solution enables the trusted third-party AF to provide application policy information to the operator's Service Hosting Environment. This solution also enables automatic management of application resources.

#### 6.9.1.2	Network slice quota management capability exposure

Figure 6.9.1.2-1 illustrates the network slice quota management process to address the key issue 10 for managing trusted third-party owned application(s).

Pre-conditions:

1.	The network slice enabler layer is capable to interact with NEF.

# Figure 6.9.1.2-1 Network slice quota management process

1.	The VAL server initiates network slice quota management request towards the NSCE server. The request includes VAL service ID, Slice identity, identity of the type of the quota, specific type of action to take (e.g. release low priority users as identified by 5GC, release list of users as idenfied by the VAL server, etc) and optionally list of UEs on which specific action to apply. The message also includes unique identity for the request in order to update or cancel the action based on change of the requirement or service provider's policy.

2.	Upon receiving the request from the VAL server to manage the network slice quota, the NSCE server authorises the VAL server and if VAL server is authorized, the NSCE server subscribes to the network slice quota events. The NSCE server includes possible actions (e.g. maximum number of UEs per slice is reached, etc) expected from 3GPP core network upon reaching the network slice quota threshold.

Editor's note:	The APIs to subscribe to the network slice quota related events is to be defined in SA2.

3.	The NSCE server sends the network slice quota management response specifying the result.

4.	Upon receiving notification from the 5GC/NEF about network slice quota threshold, the NSCE server may request the 5GC to perform specific action if not specified already.

### 6.9.2	Solution evaluation

Editor's Note:	This subclause will evaluate the solution.

## 6.10	Solution 10: Network slice application policy management capability

### 6.10.1	Solution description

#### 6.10.1.1	General

This solution aims to address the issues identified in Key Issue 10.

This solution provides a possible procedure to illustrate the process of the VAL server requesting to manage network slice through NSCE server when reaching UEs slice quota threshold. This solution enables the trusted third-party AF to provide network slice access management policy based on the UE's priority, contract qualities level, etc. This solution also helps to avoid traffic loss in case of slice congestion and make adjustment beforehand considering the application policy from AF.

#### 6.10.1.2	Network slice application policy management capability

Figure 6.10.1.2-1 illustrates the application policy capability process to address the key issue 10 described in clause 5.10.

Pre-conditions:

1.	The VAL server is authorized to use the slice event reporting exposure and slice adaptation trigger capability.

2.	The network slice enabler layer is capable to interact with NEF/NWDAF/NSACF and OAM system.

3.	NSCE has added UEs to different slices based on UE priority, QoS reservation. High priority UEs with dedicated reserved resources are in one slice.

# Figure 6.10.1.2-1 Network slice application policy management process

1.	The VAL server initiates network slice event reporting exposure request towards the NSCE server. The request includes VAL service ID, Event Filter (such as slice identity, event type, etc.), Event Reporting filter (such as whether the notification is threshold based or periodical). The message also includes notification threshold or periodicity based on the Event Reporting filter settings.

2-3.	Upon receiving the request from the VAL server to manage the network slice quota event reporting, the NSCE server authorises the VAL server and if VAL server is authorized, the NSCE server subscribes to the network slice quota (the number of UE or PDU sessions) events, including the slice identity (S-NSSAI) and threshold/periodicity information. According to 3GPP TS 23.502 [4], there can be multiple NSACFs serving the same network slice and different threshold can be set based on the policy of trusted third-party AF. The event subscription procedure is described in clause 4.15.3.2.10 of TS 23.502 [4], and the APIs defined in clause 6.2 of TS 29.536 [22] can be utilized.

4.	The NSCE server receives the network slice event notification response specifying the result after 5GC confirms the subscription.

5-6.	When the event (e.g. the number of UE or PDU sessions has reached the threshold for specific slice) happens, the notification is sent from 5GC to NSCE server including the Event Reporting information (such as percentage of the maximum number of UEs or percentage of the maximum number of the PDU Sessions established on the network slice). The event notification procedure is specified in clause 4.15.3.2.10 of TS 23.502 [4].

7.	The notification is forwarded from NSCE server to VAL server including the Event ID, Event filter and Event reporting information.

8.	The VAL server initiates network slice adaptation trigger request to the NSCE server.

9.	The NSCE server retrieves the network slice related status information from 5GC and OAM. For OAM system, the services defined in clause 11.3 of TS 28.532 [8] and clause 5 of TS 28.552 [12] can be utilized. For CN functions, the services of Nnwdaf_AnalyticsInfo service defined in clause 7.3 of TS 23.288 [17] can be utilized.

Editor's Note: Describing what OAM parameters have impact on maximum number of UEs/maximum PDU sessions per slice is FFS.

10.	The NSCE server makes network slice adaptation analysis on network status information from 5GS and make slice adjustment decision. NSCE server can make slice lifecycle management operations for specific slice as specified in step 11.

11. The NSCE server determines whether and what network slice LCM operations should be taken and makes the decision(s)/recommendation(s) (e.g. to allocate more RAN resources for UEs with high priority), such as modifyNsi request as specified in TS 28.531 [5].

### 6.10.2	Solution evaluation

Editor's Note:	This subclause will evaluate the solution.

## 6.11	Solution 11: Communication service management exposure

### 6.11.1	Solution description

#### 6.11.1.1	General

This solution aims to address the issues identified in Key Issue 5.

This solution provides a procedure for communication services management including the communication service lifecycle and communication assurance performed by the network slice capability enablement server.

To enable the verticals to create/modify/disengagement a communication service, the NSCE layer helps the VAL server to subscribe the slice services to allocate the network slices to fulfil the communication services. Also the NSCE server is able to perform the network slice related analyses to help the verticals to assure the communication service.

#### 6.11.1.2	Use Case of Communication service lifecycle management

Figure 6.11.1.2-1 illustrates one possible procedure of communication service management to address the communication service lifecycle management and communication service assurance

Pre-conditions:

1.	The VAL client ordered slice services to offer its communication services.

2.	The network slice enabler layer is registered/capable for interacting with 5GS such as triggering network slice LCM operations.

# Figure 6.11.1.2-1: communication service creation

1.	The VAL server sends a request to NSCE server to create a communication service, e.g., the VAL server requests to create a video service in a future factory, the requirements of the video service (for instance, the service ID, the number of the UEs, the coverage area of the video service, the mobility information of the UEs, the resolution of the video service) is included in this request.

2.	NSCE server translates the communication service requirements (e.g., for a video service, the mobility information and the resolution of the video) then determines the network SLA (e.g., dLtThptPerSlice, uLtThptPerSlice, latency as defined in serviceProfile TS 28.541[14]). The NSCE may perform the translation by pre-configured industry profiles or by KQI-KPI translation algorithms which are out scope of standard.

3.	NSCE server initiates the Slice Service subscription procedures by utilizing the management service of network slice creation as defined in clause 6.1, TS 28.531[5] exposed by EGMF defined in SA5. The slice creation request may fail due to the shortage of network resources.

4.	NCSE server sends the communication service creation response to VAL server. If the slice creation failed in step3, the NSCE server may ask the VAL sever to modify the service requirements.

# Figure 6.11.1.2-2: communication service modification

1.	VAL server sends the communication service reconfiguration request to the NSCE server to reconfigure the properties of the communication service if the SLA is not able to satisfy current service requirements (e.g., reconfigure the resolution requirements of the communication service) or the VAL server predicts that service requirements changes sometime in the future (e.g., some new cameras are to be supported).

2.	NSCE server translates the communication requirements according the reconfigured communication properties then updates the network SLA.

3.	NSCE server initiates the Slice Service update procedures by utilize the management service of network slice modification as defined in clause 6.1, TS 28.531[5] exposed by EGMF defined in SA5.

4.	NCSE server sends the communication service modification response to VAL server.

# Figure 6.11.1.2-3: communication service disengagement

1.	The VAL server sends a request to NSCE server to disengage the communication service when the communication service ends.

2.	NSCE server initiates the Slice Service de-allocation procedures by utilizing the management service of network slice de-allocation as defined in clause 6.1, TS 28.531[5] exposed by EGMF defined in SA5.

3.	NCSE server sends the communication service disengagement response to VAL server.

#### 6.11.1.3	Communication service creation

# Figure 6.11.1.3-1: communication service creation

Figure 6.11.1.3-1 shows the APIs of service subscription of communication service creation as step 1 and step 4 of Figure 6.11.1.2-1.

#### 6.11.1.4	Communication service modification

# Figure 6.11.1.4-1: communication service reconfiguration

Figure 6.11.1.4-1 shows the APIs of service subscription of communication service reconfiguration as step 1 and step 4 of Figure 6.11.1.2-2.

#### 6.11.1.5	Communication service disengagement

Figure 6.11.1.5-1: communication service reconfiguration

Figure 6.11.1.5-1 shows the APIs of service subscription of communication service reconfiguration as step 1 and step 3 of Figure 6.11.1.2-3.

## 6.X	Solution #X: <title>

Editor's Note:	Provide a suitable title for the solution.

### 6.X.1	Solution description

Editor's Note:	This subclause will describe the solution.

### 6.X.2	Solution evaluation

Editor's Note:	This subclause will evaluate the solution.

# 7	Overall evaluation

Editor's Note:	This clause will provide evaluation of different solutions.

# 8	Conclusions

Editor's Note:	This clause is intended to list conclusions that have been agreed during the course of the study item activities.

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-07 | SA6#44 |  |  |  |  | TR skeleton | 0.0.0 |
| 2021-07 | SA6#44 |  |  |  |  | TR Skeleton agreed in SA6#44: S6-211848 Implemented pCRs approved in SA6#44: S6-211849，S6-211850，S6-211851，S6-211782 Editorial changes by the rapporteur | 0.1.0 |
| 2021-09 | SA6#45 |  |  |  |  | Implemented pCRs approved in SA6#45: S6-212161，S6-212162，S6-212080，S6-212079，S6-211949，S6-211965，S6-211967，S6-212163，S6-212144，S6-212069 Editorial changes by the rapporteur | 0.2.0 |
| 2021-10 | SA6#45-bis |  |  |  |  | Implemented pCRs approved in SA6#45-bis: S6-212202, S6-212204, S6-212207, S6-212210, S6-212474, S6-212400 Editorial changes by the rapporteur | 0.3.0 |
| 2021-11 | SA6#46 |  |  |  |  | Implemented pCRs approved in SA6#46: S6-212765, S6-212761, S6-212763, S6-212597, S6-212536, S6-212546, S6-212726, S6-212727, S6-212733, S6-212736, S6-212764, S6-212724, S6-212725, S6-212755, S6-212770; Editorial changes by the rapporteur. | 0.4.0 |
| 2022-02 | SA6#47 |  |  |  |  | Implemented pCRs approved in SA6#46: S6-220421, S6-220098, S6-220338, S6-220305, S6-220097, S6-220307, S6-220445, S6-220446, S6-220447, S6-220126, S6-220334, S6-220448, S6-220337, S6-220339, S6-220308, S6-220322. Editorial changes by the rapporteur. | 0.5.0 |