| 3GPP TS 24.538 V0.2.1 (2021-12) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Enabling MSGin5G Service; Protocol specification; (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

# Contents

Foreword	6

Introduction	7

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	9

3.1	Terms	9

3.2	Symbols	9

3.3	Abbreviations	9

4. General description	9

5	Functional entities	11

5.1	MSGin5G Client	11

5.2	MSGin5G Server	11

6	MSGin5G Procedures	12

6.1	General	12

6.2	Configuration	12

6.2.1	MSGin5G UE Configuration	12

6.2.1.1	General	12

6.2.1.2	Procedure at MSGin5G Client	12

6.2.1.3	Procedure at MSGin5G Server	13

6.2.2	Constrained device Configuration	13

6.2.2.1	Procedure at Gateway MSGin5G UE	13

6.2.2.2	Procedure at Constrained device	13

6.3	Registration	13

6.3.1	MSGin5G UE Registration	13

6.3.1.1	Procedure at MSGin5G Client	13

6.3.1.1.1	MSGin5G UE registration	13

6.3.1.1.2	MSGin5G UE de-registration	14

6.3.1.2	Procedure at MSGin5G Server	15

6.3.1.2.1	MSGin5G UE registration	15

6.3.1.2.2	MSGin5G UE de-registration	15

6.3.2	Constrained device registration to use gateway MSGin5G UE	16

6.3.2.1	Procedure at Gateway MSGin5G UE	16

6.3.2.1.1	Constrained device registration to use gateway MSGin5G UE	16

6.3.2.1.2	Constrained device de-registration to use gateway UE	16

6.3.2.1	Procedure at Constrained device	16

6.3.2.1.1	Constrained device registration to use gateway MSGin5G UE	16

6.3.2.1.2	Constrained device de-registration to use gateway UE	16

6.4	MSGin5G Message delivery	16

6.4.1	Procedures between MSGin5G UE and MSGin5G Server	16

6.4.1.1	Procedure at MSGin5G Client	17

6.4.1.1.1	General	17

6.4.1.1.2	Sending of an MSGin5G message	17

6.4.1.1.3	Sending of an aggregated MSGin5G message	18

6.4.1.1.4	Sending of an MSGin5G message delivery status report	19

6.4.1.1.5	Sending of a aggregated MSGin5G message delivery status report	20

6.4.1.1.6	Reception of an MSGin5G message	20

6.4.1.1.7	Reception of a aggregated MSGin5G message	21

6.4.1.1.8	Reception of an MSGin5G message delivery status report	21

6.4.1.1.9	Reception of a aggregated MSGin5G message delivery status report	22

6.4.1.2	Procedure at MSGin5G Server	22

6.4.1.2.1	General	22

6.4.1.2.2	Reception of an MSGin5G message	22

6.4.1.2.3	Reception of an aggregated MSGin5G message	22

6.4.1.2.4	Reception of an MSGin5G delivery status report	23

6.4.1.2.5	Reception of an aggregated MSGin5G delivery status report	23

6.4.1.2.6	Sending of an MSGin5G message	23

6.4.1.2.7	Sending of an aggregated MSGin5G message	23

6.4.1.2.8	Sending of an MSGin5G delivery status report	23

6.4.1.2.9	Sending of a aggregated MSGin5G delivery status report	23

6.4.2	Procedures between Constrained device and gateway MSGin5G UE	23

6.4.2.1	Procedure at Gateway MSGin5G UE	23

6.4.2.1.1	Sending of an MSGin5G message to Constrained device	23

6.4.2.1.2	Reception of an MSGin5G message from Constrained device	23

6.4.2.2	Procedure at Constrained device	23

6.4.2.2.1	Sending of an MSGin5G message to Gateway MSGin5G UE	23

6.4.2.2.2	Reception of an MSGin5G message from Gateway MSGin5G UE	24

6.5	MSGin5G Message Segmentation and Reassembly	24

6.5.1	Procedure at MSGin5G Client	24

6.5. 2	Procedure at MSGin5G Server	24

6.6	Messaging Topic Subscription	24

6.6.1	General	24

6.6.2	Messaging Topic Subscription Procedure at MSGin5G Client	24

6.6.3	Procedure at MSGin5G Server	25

6.7	Usage of Network Capabilities	25

6.7.1	General	25

6.7.2	UE reachability status monitoring	26

6.7.2.1	Procedure at MSGin5G Client	26

6.7.2.2	Procedure at MSGin5G Server	26

6.7.3	MSGin5G device triggering	26

6.7.3.1	Procedure at MSGin5G Client	26

6.7.3.2	Procedure at MSGin5G Server	26

6.8	Usage of SEAL	26

6.8.1	General	26

6.8.2	Configuration management service	26

6.8.2.1	Procedure at MSGin5G Client	26

6.8.2.2	Procedure at MSGin5G Server	26

6.8.3	Group management service	26

6.8.3.1	Procedure at MSGin5G Client	26

6.8.3.2	Procedure at MSGin5G Server	27

7	Coding	27

7.1	General	27

7.2	MSGin5G UE Configuration data	27

7.2.1	General	27

7.2.2	Application unique ID	27

7.2.3	Structure	27

7.2.4	XML schema	27

7.2.4.1	General	27

7.2.4.2	XML schema for MSGin5G specific extensions	27

7.2.5	Data semantics	28

7.2.6	MIME types	28

7.3	MSGin5G message structure	28

Annex A (informative):	Change history	29

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

# 1	Scope

The present document specifies protocols for supporting MSGin5G services as specified in 3GPP TS 23.554 [2] for:

1.	communication between the MSGin5G UE and the MSGin5G Server over the MSGin5G-1 interface.

2.	communication between the MSGin5G UE unconstrained device and gateway MSGin5G UE over the MSGin5G-5 and MSGin5G-6 interfaces.

The present specification defines the usage and interactions of the MSGin5G Service with SEAL services.

The present specification defines the usage of the necessary 5GC Network Capabilities, e.g. device triggering.

The present specification also defines the message format, message contents, error handling and system parameters applied by the protocols for the MSGin5G Service.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.554: "Application architecture for MSGin5G Service; Stage 2;".

[3]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals".

[4]		IETF RFC 7641: "Observing Resources in the Constrained Application Protocol (CoAP)".

[5]		IETF RFC 7252: "The Constrained Application Protocol (CoAP)".

[6]		3GPP TS 24.546: "Configuration management - Service Enabler Architecture Layer for Verticals (SEAL); Protocol specification".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

CAPIF	Common API Framework for northbound APIs

MSG-C	MSGin5G Client

MSG-S	MSGin5G Server

NIDD	Non IP Data Delivery

SEAL	Service Enabler Architecture Layer for Verticals

# 4. General description

The MSGin5G Service (message service for MIoT over 5G System) is basically designed and optimized for massive IoT device communication including thing-to-thing communication and person-to-thing communication. The MSGin5G Service provides messaging communication capability in 5GS including the following message communication models:

1.	Point-to-Point message;

2.	Application-to-Point message/ Point-to-Application message;

3.	Group message;

4,	Broadcast message.

The MSGin5G Service support the message exchanging between the following UE types:

1.	MSGin5G UE:

1)	light weight constrained devices (e.g. sensors, actuators) and

2)	unconstrained devices with advanced capabilities (e.g. washing machine, micro-ovens).

2.	Legacy 3GPP UE.

3.	Non-3GPP UE.

The MSGin5G Client contained in the MSGin5G UE communicates with the MSGin5G Server over the MSGin5G-1 interface (see 3GPP TS 23.554 [2]). CoAP specified in IETF RFC 7252 [5] is used as the basic transport protocol of MSGin5G service in this reference point MSGin5G-1 and shall be supported by the MSGin5G Client and MSGin5G Server.

An MSGin5G UE-1 may be constrained devices which do not have enough capability to communicate with MSGin5G Server. If allowed by configuration, an unconstrained device MSGin5G UE-2 may act as a UE Message Gateway to MSGin5G UE-1. In this scenario, the MSGin5G UE-1 communicates with the MSGin5G UE-2 over the MSGin5G-5 and/or MSGin5G-6 interfaces (see 3GPP TS 23.554 [2]).

Additionally, the MSGin5G Client(s) may interacts with SEAL Clients over the SEAL-C reference point specified for each SEAL service as specified in 3GPP TS 23.434 [3]. The MSGin5G Server(s) may interacts with SEAL Servers over the SEAL-S reference point specified for each SEAL service as specified in 3GPP TS 23.434 [3]. The interaction between a SEAL Client and the corresponding SEAL Server is supported by SEAL-UU reference point specified for each SEAL service as specified in 3GPP TS 23.434 [3].

By means of using the MSGin5G-1 interface, the following aspects can be provided:

1.	MSGin5G UE registration and de-registration towards the MSGin5G Server;

2.	MSGin5G message delivery and MSGin5G message delivery status report;

3.	Messaging Topic Subscription; and

4.	usage of Network Capabilities, e.g. UE reachability status monitoring and device triggering.

By means of using the MSGin5G-5 and/or MSGin5G-6 interfaces, the following aspects can be provided:

1.	Constrained device registration and de-registration towards the MSGin5G Server by using gateway MSGin5G UE.

2.	The exchanging of MSGin5G message and MSGin5G message delivery status report between constrained device and MSGin5G Server by using gateway MSGin5G UE.

# 5	Functional entities

## 5.1	MSGin5G Client

An MSGin5G Client acts as client-side functionality for MSGin5G UE configuration, registration and message delivery.

The functionalities of MSGin5G Client include:

-	supporting registration to an MSGin5G Server;

-	supporting configuration required to use MSGin5G Service;

-	constructing MSGin5G message when requested by an Application Client;

-	delivering MSGin5G message payload to the targeted Application Client; and

-	exchanging MSGin5G messages with an MSGin5G Server.

## 5.2	MSGin5G Server

An MSGin5G Server functional entity provides server-side functionality for configuration, registration and message delivery.

The following functionalities of MSGin5G Server need to be considered in current document:

-	exchanging MSGin5G messages with MSGin5G Client;

-	routing MSGin5G messages based on UE Service ID;

-	supporting transport level protocol selection and conversion for exchanging MSGin5G messages with MSGin5G UE;

-	to resolve the MSGin5G Group Service ID to determine the members of the Group specified in 3GPP TS 23.434 [3];

-	supporting MSGin5G message segmentation according to service provider's policy;

-	supporting MSGin5G UE configuration procedures as specified in TS 23.434 [3] or communicating with the SEAL Configuration Management Server to provide MSGin5G configuration data on a UE to be ready for the MSGin5G Service; and

-	managing MSGin5G UE information related to the MSGin5G Service, such as MSGin5G Client availability.

# 6	MSGin5G Procedures

## 6.1	General

## 6.2	Configuration

### 6.2.1	MSGin5G UE Configuration

#### 6.2.1.1	General

MSGin5G UE Configuration is based on the Configuration management functionality specified in TS 23.434 [3] and TS 24.546 [6].

#### 6.2.1.2	Procedure at MSGin5G Client

The MSGin5G UE should support the Configuration management client functionality as specified in 3GPP TS 23.546 [6] in or outside the MSGin5G Client. The configuration management client may be collocated with MSGin5G Client or it can be separate as per 3GPP TS 23.554 [2].

If the Configuration management client function is not collocated with the MSGin5G Client, the MSGin5G Client should use SEAL-C interface to interact with Configuration management client function for MSGin5G UE configuration.

The MSGin5G UE configuration procedures at the Configuration management client function is based on the procedures in clause 6.2.3.1 of 3GPP TS 24.546 [6], in the procedures:

a)	the Configuration management client function on the MSGin5G UE acts as SCM-C;

b)	the Configuration management server function at the server-side acts as SCM-S;

c)	in the Request-URI, the "XCAP Root" is the URI of Configuration management server function;

d)	in the Request-URI, the "auid" is the unique service identifier of MSGin5G service; and

e)	the parameters serialized into a JavaScript Object Notation (JSON) structure includes:

1)	MSGin5G UE ID as specified in 3GPP TS 23.554 [2]; and

2)	optionally, other related information (e.g. device type, device vendor, etc).

Upon receiving the requested MSGin5G UE configuration data, the Configuration management client function shall submit the configuration data to MSGin5G Client by SEAL-C interface. The MSGin5G Client shall store the configuration data, including MSGin5G UE Service ID, the address of MSGin5G Server and other available MSGin5G Service specific informations.

#### 6.2.1.3	Procedure at MSGin5G Server

The Configuration management server functionality as specified in 3GPP TS 23.546 [6] may be collocated with MSGin5G Server or it can be separate as per 3GPP TS 23.554 [2].

The MSGin5G UE configuration procedures at the Configuration management server function is based on the procedures in clause 6.2.3.2 of 3GPP TS 24.546 [6], in the procedures, the configuration management server function act as SCM-S.

### 6.2.2	Constrained device Configuration

This clause covers the procedures in MSGin5G-5 and/or MSGin5G-6.

#### 6.2.2.1	Procedure at Gateway MSGin5G UE

#### 6.2.2.2	Procedure at Constrained device

## 6.3	Registration

Authentication procedures may be added to this clause.

### 6.3.1	MSGin5G UE Registration

This clause covers the procedures in MSGin5G-1.

#### 6.3.1.1	Procedure at MSGin5G Client

##### 6.3.1.1.1	MSGin5G UE registration

After the MSGin5G UE receives the UE Service ID, in order to register MSGin5G UE to the MSGin5G Server, the MSGin5G Client shall send a CoAP POST request message to the MSGin5G Server to according to procedures specified in IETF RFC 7252 [5]. In the CoAP POST request message, the MSGin5G Client:

a)	shall set the "T" field in the CoAP header to 0 to indicate acknowledge message required;

b)	shall include the MSGin5G Server address in the Option header of the CoAP message and set the Option header to a corresponding value, e.g. if the MSGin5G Server address is a URI, the Uri-Path Option is set to the value of such URI;

c)	shall set the "Content-Format" element to "50" to indicate the format of the CoAP payload is "application/json"; and

d)	shall include the following information elements in the CoAP payload in the MSGin5G registration request:

1)	the "Message Type" element to indicate a registration message;

2)	the "UE Service ID" element to indicate the MSGin5G UE initiating registration procedure;

3)	the "UE Credential Information" element to indicate the authentication type and related information;

# Editor's note:	The details of authentication type and related information are FFS.

4)	optionally, the "MSGin5G UE ID" element to indicate the MSGin5G UE hosting the MSGin5G Client and the "MSGin5G Client Ports" element to indicate that the MSGin5G Client listens on for device triggers from the MSGin5G Server; and

5)	optionally, the "MSGin5G Client Communication Availability" element to informs the MSGin5G Server whether the client has a specific application-level schedule/periodicity to its MSGin5G communications, which may be used in conjunction with UE reachability monitoring to determine whether and when MSGin5G communications are attempted. This element:

i)	shall include the "Scheduled communication time" element to indicate the time when the UE becomes available for communication;

ii)	shall include the "Communication duration time" element to indicate the duration time of periodic communication;

iii)	may include the "Periodic communication indicator" element to identify whether the client communicates periodically or not;

iv)	shall include the "Periodic communication interval" element to indicate the interval Time of periodic communication in case of periodic communications if "Periodic communication indicator" element is included;

v)	may include the "Data size indication" element to indicate the expected data size to be exchanged during the communication duration; and

vi)	may include the "Store and forward option" element to indicate the UE does not request store and forward services for incoming MSGin5G requests.

##### 6.3.1.1.2	MSGin5G UE de-registration

The MSGin5G Client initiates an CoAP POST request message to de-register from the MSGin5G Server. In the CoAP POST request message, the MSGin5G Client:

a)	shall set the "T" field in the CoAP header to 0 to indicate acknowledge message required;

b)	shall include the MSGin5G Server address in the Option header of the CoAP message and set the Option header to a corresponding value, e.g. if the MSGin5G Server address is a URI, the Uri-Path Option is set to the value of such URI;

c)	shall set the "Content Format" element to "50" to indicate the format of the CoAP payload is "application/json"; and

d)	includes the following information elements in the CoAP payload in the MSGin5G de-registration request:

1)	the "Message Type" element to indicate a de-registration message;

2)	the "UE Service ID" element to indicate the MSGin5G UE initiating de-registration procedure; and

3)	the "UE Credential Information" element to indicate the authentication type and related information.

# Editor's note:	The details of authentication type and related information are FFS.

#### 6.3.1.2	Procedure at MSGin5G Server

##### 6.3.1.2.1	MSGin5G UE registration

Upon reception of registration message carried by the CoAP POST request message, the MSGin5G Server shall verifies the security credentials according to "UE Credential Information" element. After a successful verification, the MSGin5G Server:

a)	shall store the UE Service ID included in bullet 2) and the MSGin5G Client Profile information included in bullet 4) and 5) as specified in subclause 6.3.1.1.1; and

b)	shall generate a CoAP 2.01 (Created) response or CoAP 2.04 (Change) response including the following parameters:

1)	the CoAP "Message ID" element and the "Token" element with the same values with those in the CoAP POST request message for registration;

2)	optionally, the MSGin5G Client address in the Option header of the CoAP response message and set the Option header to a corresponding value, if it is provided in the payload of CoAP POST request message; and

3)	the "Content-Format" element with "50" to indicate the format of the CoAP payload is "application/json" and the CoAP payload including:

i)	the "UE Service ID" element to indicate the MSGin5G UE initiating registration procedure; and

ii)	the "Registration result" element to indicate whether the registration is success or failure.

# Editor's note:	How to verify the security credentials is FFS.

##### 6.3.1.2.2	MSGin5G UE de-registration

Upon reception of deregistration message carried by the CoAP POST request message from a MSGin5G UE, the MSGin5G Server shall verifies the security credentials according to "UE Credential Information" element. After a successful verification, the MSGin5G Server:

a)	shall delete the registration information of the MSGin5G UE and any applicable MSGin5G Client Profile information that it has stored; and

b)	shall generate a CoAP 2.04 (Change) response including the following parameters:

1)	the CoAP "Message ID" element and the "Token" element with the same values with those in the CoAP POST request message for deregistration;

2)	optionally, the MSGin5G Client address in the Option header of the CoAP response message and set the Option header to a corresponding value, if it is provided in the payload of CoAP POST request message; and

3)	the "Content-Format" element with "50" to indicate the format of the CoAP payload is "application/json" and the CoAP payload including:

i)	the "UE Service ID" element to indicate the MSGin5G UE initiating registration procedure; and

ii)	the "De-registration result" element to indicate whether the registration is success or failure.

# Editor's note:	How to verify the security credentials is FFS.

### 6.3.2	Constrained device registration to use gateway MSGin5G UE

This clause covers the procedures in MSGin5G-5 and/or MSGin5G-6.

#### 6.3.2.1	Procedure at Gateway MSGin5G UE

##### 6.3.2.1.1	Constrained device registration to use gateway MSGin5G UE

##### 6.3.2.1.2	Constrained device de-registration to use gateway UE

#### 6.3.2.1	Procedure at Constrained device

##### 6.3.2.1.1	Constrained device registration to use gateway MSGin5G UE

##### 6.3.2.1.2	Constrained device de-registration to use gateway UE

## 6.4	MSGin5G Message delivery

All of the messaging scenarios, i.e. Point-to-Point message, Application-to-Point message/ Point-to-Application message, Group message and Broadcast message, are included in this clause.

### 6.4.1	Procedures between MSGin5G UE and MSGin5G Server

This clause covers the procedures in MSGin5G-1.

#### 6.4.1.1	Procedure at MSGin5G Client

##### 6.4.1.1.1	General

##### 6.4.1.1.2	Sending of an MSGin5G message

In order to send an MSGin5G message, the MSGin5G Client shall compares the size of the received message from the application client to the maximum allowed MSGin5G message segmentation size. If the size exceeds, the MSGin5G Client shall segment the MSGin5G message into a set of segmented MSGin5G messages such that each segmented MSGin5G message can fit within the maximum allowed MSGin5G message segmentation size. For each segmented MSGin5G message, the steps listed below shall be processed individually.

The MSGin5G Client shall send the MSGin5G message in an CoAP POST request message according to procedures specified in IETF RFC 7252 [5]. In the CoAP POST request message, The MSGin5G Client:

a)	shall set the "T" field in the CoAP header to 0 if delivery status report from the recipient is requested, i.e. indicates this message is the type of Confirmable, to ensure the application layer delivery status report;

b)	shall include the MSGin5G Server address in an CoAP Option, e.g. if the MSGin5G Server address is a URI, include a Uri-Path Option with the value of the URI;

c)	shall set the CoAP Content-Format to "50", i.e. application/json; and

d)	shall include the information elements specified in 3GPP TS 23.554 [2] in the CoAP payload encoded in JSON format:

# Editor’s note:	the JSON CoAP payload is to be specified in clause 7.

1)	shall include an "MSGin5G service identifier" element to indicate that this CoAP POST request message is used for MSGin5G service;

2)	shall include an "Originating UE Service ID" element set to the UE which requests the sending of the MSGin5G message;

3)	shall include a "Recipient UE Service ID/AS Service ID" element if the recipient(s) is(are) MSGin5G UE/Non-MSGin5G UE or Application Server;

4)		shall include a "Group Service ID" element if the recipient is an MSGin5G Group;

5)	shall include a "Broadcast Area ID" element if the message needs to be broadcast;

6)	shall include a "Messaging Topic" element if this message will be distributed based on message topic. This element shall not present in other message scenarios;

# NOTE:	In an MSGin5G Message request, only one of these IEs listed in 3) to 6) shall be included.

7)	may include one or more "Application ID" element(s) to indicate the application(s) for which the payload is intended;

8)	shall include a "Message ID" which is globally unique within the MSGin5G service;

9)	shall include a "Security credentials" element which is required by the MSGin5G Server;

# Editor's note:	the Security credentials element is a placeholder for SA3 security information.

10)	may include a "Delivery status required" element if delivery acknowledgement from the recipient is requested;

11)	may include a "Priority type" element to indicate the application priority level requested for this message;

12)	may include a "Message is segmented" element to indicate that this message is part of a segmented message;

13)if "Message is segmented" element is included, shall include a "Segmentation set identifier" element to indicate that this segmented message is associated within a set of segmented messages , all segmented messages associated with the same MSGin5G message are assigned the same unique identifier;

14)	if "Message is segmented" element is included and this message is the first segment of the MSGin5G message, shall include a "Total number of message segments" element to indicate the total number of segments for the MSGin5G message;

15)	if "Message is segmented" element is included, shall include a "Message segment number" element to indicate segmented message number of each segmented message within a set of segmented messages;

16)			if "Message is segmented" element is included and this message is the last segment of the MSGin5G message, shall include a "Last segment flag" element to indicate that this segmented message is the last segment in the set of segmented messages;

17)shall include a "Store and forward flag" element to indicate whether store and forward services are requested for this message;

18)	if store and forward services are requested, may include a "Store and forward parameters" element to carry the parameters used by MSGin5G Server for providing store and forward services. The "Store and forward parameters":

i)	may include a "Message expiration time" element to indicate message expiration time used for providing store and forward services if the destination is not available for communications; and

ii)	may include a "Application specific store and forward information" element to carry the information used by MSGin5G Server for handling store and forward, e.g. a delivery time/date; and

19)	may include a "Payload" element specified in 3GPP TS 23.554 [2] in the CoAP payload and located it after the MSGin5G header to carry the payload of this message.

##### 6.4.1.1.3	Sending of an aggregated MSGin5G message

Before the sending of an MSGin5G message, the MSGin5G Client shall check if aggregation is allowed for this message, check the message data size, and the priority level to determine if the message can be aggregated. For example, the MSGin5G Client finds that the messages have small payload size when compared to the maximum segment size that can be transmitted over CoAP and are not high priority messages, which could be sent as per scheduling policy towards a selected target. The MSGin5G Client can decide to continue aggregating messages until optimal use of segment size before sending message towards MSGin5G Server.

If the message can be aggregated, the MSGin5G Client aggregates multiple MSGin5G message requests intended for a selected target and sends the aggregated message in a single CoAP POST request message. The sending of the CoAP POST request message shall follow the procedures specified in clause 6.4.1.1.2 with the clarifications listed below:

a)	The MSGin5G Client should not segment the aggregated message, so in step d) of clause 6.4.1.1.2, the "Message is segmented", "Segmentation set identifier", "Total number of message segments", "Message segment number" and "Last segment flag" elements should not be included.

b)	In addition to the step d) of clause 6.4.1.1.2, the MSGin5G Client should include a "Number of individual messages" element in this message to indicate the total number of messages which are aggregated into this single message.

c)	In addition to the step d) of clause 6.4.1.1.2, the MSGin5G Client should include a "List of individual messages" element in this message. Each child element in this "List of individual messages" element contains information elements listed below:

1)	"Message ID" of the individual message;

2)	"Payload";

3)	one of more optional "Application ID" element(s);

4)	an optional "Delivery status required" element; and

5)	an optional "Priority type" element.

d)	The MSGin5G Client should not include the "Payload" element, i.e. the 19) in step e) of clause 6.4.1.1.2 shall not be processed.

##### 6.4.1.1.4	Sending of an MSGin5G message delivery status report

In order to send a MSGin5G message delivery status report, the MSGin5G Client shall send an CoAP POST request according to procedures specified in IETF RFC 7252 [5]. In the CoAP POST request, the MSGin5G Client:

a)	shall sets the "T" field in the CoAP header to 0, i.e. indicates that this message is the type of Confirmable, to ensure the MSGin5G message delivery status report can be received by the originator of the receiving MSGin5G message;

b)	shall include the MSGin5G Server address in an CoAP Option, e.g. if the MSGin5G Server address is a URI, include a Uri-Path Option with the value of the URI;

c)	shall set the CoAP Content-Format to "50", i.e. application/json; and

d)	shall include the information elements specified in 3GPP TS 23.554 [2] in the CoAP payload encoded in JSON format:

# Editor’s note:	the JSON CoAP payload is to be specified in clause 7.

1)	shall include an "MSGin5G service identifier" element to indicate that this CoAP POST request message is used for MSGin5G service;

2)	shall include an "Originating UE Service ID" element set to the UE which requests the sending of the MSGin5G message delivery status report;

3)	shall include a "Recipient UE Service ID/AS Service ID" element if the recipient(s) is(are) MSGin5G UE/Non-MSGin5G UE or Application Server, this is the sender of the message that this message delivery status report is for;

4)	shall include the "Message ID" element copied from the MSGin5G message that is being acknowledged;

5)	may include the "Security credentials" which is required by the MSGin5G Server;

# Editor's note:	the Security credentials element is a placeholder for SA3 security information.

6)	shall include a "Delivery Status" element to carry the delivery status description, the delivery status can be success or failure in delivery; and

7)	may include a "Failure Cause" element indicates the failure reason if the delivery status is failure;

##### 6.4.1.1.5	Sending of a aggregated MSGin5G message delivery status report

The MSGin5G Client can aggregate multiple MSGin5G message delivery status reports into one single message. The MSGin5G Client shall check whether the MSGin5G message delivery status reports can be aggregated as specified in clause 6.4.1.1.3.

If the MSGin5G message delivery status reports can be aggregated, the MSGin5G Client aggregates MSGin5G message delivery status reports intended for a selected target and sends the aggregated MSGin5G message delivery status reports in a single CoAP POST request message. The sending of the CoAP POST request message shall follow the procedures specified in clause 6.4.1.1.4 with the clarifications listed below:

a)	In step d) of clause 6.4.1.1.4, the "Delivery Status" element and the "Failure Cause" element should not be included.

b)	In addition to the step d) of clause 6.4.1.1.4, the MSGin5G Client should include a "Number of individual messages" element in this message to indicate the total number of MSGin5G message delivery status reports which are aggregated into this single message.

c)	In addition to the step d) of clause 6.4.1.1.4, the MSGin5G Client should include a “"List of individual messages" element in this message. Each child element in this "List of individual messages" element contains information elements listed below:

1)	"Message ID" of the individual MSGin5G message delivery status reports which is copied from the MSGin5G message that is being acknowledged;

2)	"Delivery Status" element; and

3)	an optional "Failure Cause" element.

##### 6.4.1.1.6	Reception of an MSGin5G message

Upon receiving an CoAP POST request containing the MSGin5G Service identifier, if the "Number of individual messages" element and "List of individual messages" element are not be included, the MSGin5G Client shall handle the CoAP POST request according to procedures specified in IETF RFC 7252 [5] with the clarifications listed below:

a)	The MSGin5G Client shall check whether a "Message is segmented" element is included in the CoAP POST request. If this element is included, the MSGin5G Client shall wait until all the segmented messages have been received by checking the "Segmentation set identifier", "Total number of message segments", "Message segment number" and "Last segment flag" elements. the MSGin5G Client shall reassembles all the segmented messages into a single MSGin5G message.

b)	The MSGin5G Client shall provide the received information in the “payload” element to the application client(s) if one or more "Application ID" element(s) is(are) included. The application client(s) is(are) indicated by the "Application ID" element(s):

1)	If the Application Client is on the other MSGin5G UE-2 for which this MSGin5G Client is acting as Gateway UE, the MSGin5G Client shall send the received information to the corresponding MSGin5G UE via MSGin5G-6 (if MSGin5G Client is supported by MSGin5G UE-2) or MSGin5G-5 reference point (if MSGin5G Client is not supported by MSGin5G UE-2) as specified in clause 6.4.2.1.1.

2)	If the Application Client is on the same MSGin5G UE with the MSGin5G Client, the MSGin5G Client shall deliver the received information to the Application Client via MSGin5G-5 reference point.

NOTE:	when the Application Client and MSGin5G Client are resided on the same MSGin5G UE, the interaction in MSGin5G-5 reference point may implementation specific and is out of scope of the present document.

c)	If a "Delivery status required" element is included in the CoAP POST request, the MSGin5G Client shall send an MSGin5G message delivery status report as specified in clause 6.4.1.1.4 or 6.4.1.1.5 with the clarifications listed below:

1)	if the message delivery status is supported by the Application Client(s), the MSGin5G message delivery status report shall be sent after the delivery status information is received from the Application Client(s), and shall be generated based on this delivery status information; or

2)	if the message delivery status is not supported by the Application Client, the MSGin5G message delivery status report shall be sent immediately by the MSGin5G Client on behalf of the Application Client(s).

##### 6.4.1.1.7	Reception of a aggregated MSGin5G message

Upon receiving an CoAP POST request containing the MSGin5G Service identifier, if a "Number of individual messages" and a "List of individual messages" are included, the MSGin5G Client determines that this message is an aggregated MSGin5G message. The MSGin5G Client shall handle the CoAP POST request according to procedures specified in IETF RFC 7252 [5] with the clarifications listed below:

a)	The MSGin5G Client shall split the received aggregated message request into multiple individual MSGin5G message, each individual MSGin5G message contains the information elements included in the child element in this "List of individual messages" element.

# Editor’s note: How the MSGin5G Client splits the aggregated message is FFS.

b)	The MSGin5G Client shall handle each individual MSGin5G messages according to step b) and c) specified in clause 6.4.1.1.6.

##### 6.4.1.1.8	Reception of an MSGin5G message delivery status report

Upon receiving an CoAP POST request containing the MSGin5G Service identifier, if the "Number of individual messages" element and "List of individual messages" element are not be included and a "Delivery Status" element is included, the MSGin5G Client shall handle the CoAP POST request according to procedures specified in IETF RFC 7252 [5] with the clarifications listed below:

a)	The MSGin5G Client shall provide the received information in the "Delivery Status" element and the "Failure Cause" element (if applicable) to the application client(s) if one or more "Application ID" element(s) is(are) included. The application client(s) is(are) indicated by the "Application ID" element(s):

1)	If the Application Client on the other MSGin5G UE for which this MSGin5G Client is acting as Gateway UE, the MSGin5G Client shall send the received information to the corresponding MSGin5G UE via MSGin5G-6 (if MSGin5G Client is supported by MSGin5G UE-2) or MSGin5G-5 reference point (if MSGin5G Client is not supported by MSGin5G UE-2) as specified in clause 6.4.2.1.1.

2)	If the Application Client on the same MSGin5G UE with the MSGin5G Client, the MSGin5G Client shall deliver the received information to the Application Client via MSGin5G-5 reference point;

NOTE:	when the Application Client and MSGin5G Client are resided on the same MSGin5G UE, the interaction in MSGin5G-5 reference point may implementation specific and is out of scope of the present document.

##### 6.4.1.1.9	Reception of a aggregated MSGin5G message delivery status report

Upon receiving an CoAP POST request containing the MSGin5G Service identifier, if a "Number of individual messages" and a "List of individual messages" are included, the MSGin5G Client determines that this message is an aggregated MSGin5G message. The MSGin5G Client shall handle the CoAP POST request according to procedures specified in IETF RFC 7252 [5] with the clarifications listed below:

a)	The MSGin5G Client shall split the received aggregated message request into multiple individual MSGin5G message, each individual MSGin5G message contains the information elements included in the child element in this "List of individual messages" element.

# Editor’s note: How the MSGin5G Client splits the aggregated message is FFS.

b)	If "Delivery Status" element is included in the individual MSGin5G message, the MSGin5G Client determines that the individual MSGin5G messages are MSGin5G delivery status reports. The MSGin5G Client shall handle each individual MSGin5G delivery status report according to step a) specified in clause 6.4.1.1.8.

#### 6.4.1.2	Procedure at MSGin5G Server

##### 6.4.1.2.1	General

##### 6.4.1.2.2	Reception of an MSGin5G message

##### 6.4.1.2.3	Reception of an aggregated MSGin5G message

##### 6.4.1.2.4	Reception of an MSGin5G delivery status report

##### 6.4.1.2.5	Reception of an aggregated MSGin5G delivery status report

##### 6.4.1.2.6	Sending of an MSGin5G message

##### 6.4.1.2.7	Sending of an aggregated MSGin5G message

##### 6.4.1.2.8	Sending of an MSGin5G delivery status report

##### 6.4.1.2.9	Sending of a aggregated MSGin5G delivery status report

### 6.4.2	Procedures between Constrained device and gateway MSGin5G UE

This clause covers the procedures in MSGin5G-5 and/or MSGin5G-6.

#### 6.4.2.1	Procedure at Gateway MSGin5G UE

##### 6.4.2.1.1	Sending of an MSGin5G message to Constrained device

##### 6.4.2.1.2	Reception of an MSGin5G message from Constrained device

#### 6.4.2.2	Procedure at Constrained device

##### 6.4.2.2.1	Sending of an MSGin5G message to Gateway MSGin5G UE

##### 6.4.2.2.2	Reception of an MSGin5G message from Gateway MSGin5G UE

## 6.5	MSGin5G Message Segmentation and Reassembly

### 6.5.1	Procedure at MSGin5G Client

### 6.5. 2	Procedure at MSGin5G Server

## 6.6	Messaging Topic Subscription

### 6.6.1	General

As specified in 3GPP TS 23.554 [2], an MSGin5G Client may subscribe one or more Messaging Topic(s) on the MSGin5G Server.

The message topic subscription is based on the CoAP Observe method as specified in IETF RFC 7641 [4], the MSGin5G Client acts as an observer, the MSGin5G Server acts as a CoAP Server, the message topic is a resource to observe.

### 6.6.2	Messaging Topic Subscription Procedure at MSGin5G Client

Upon receiving a request to subscribe a messsage topic from an Application Client, MSGin5G Client shall send a CoAP GET request, as specified in IETF RFC 7641 [4], to the MSGin5G Server. In the CoAP GET request, the MSGin5G Client:

a)	shall set the "T" field in the CoAP header to the value "0" to indicate this request is the type of Confirmable;

b)	shall include the MSGin5G Server host address in the Uri-Host Option;

c)	shall include the MSGin5G Server port number in the Uri-Port Option;

d)	shall include the message topic name in the Uri-Path Option (e.g. "\top");

e)	shall include the Observe Option with the value "0" which indicates the request is for observing a resource, i.e. for subscribing a message topic;

f)	shall include the Content-Format Option with the value "50" which indicate the format of the CoAP payload is "application/json" as specified in RFC 7252 [5]; and

g)	shall include the CoAP Payload in JSON format, including the following information elements as specified in clause 8.8.1 of 3GPP TS 23.554 [2]:

1)	a "Originating UE Service ID" element set to the MSGin5G UE which requests the message topic subscription; and

2)	optionally, a "Expiration time" element which indicates the expiration time of the message topic subscription.

### 6.6.3	Procedure at MSGin5G Server

The MSGin5G Server should support parsing CoAP request as specified in RFC 7252 [5] and RFC 7641 [4].

Upon receiving a CoAP GET request from MSGin5G Client, the MSGin5G Server shall parse the CoAP headers, Options and Payload in the request to get:

a)	the value of Observe Option;

b)	the message topic from the Uri-Path Option;

c)	get the Originating UE Service ID from the Payload; and

d)	get the Expiration time from the Payload if exists in the Payload.

If the Observe Option is included in the CoAP GET request with a value "0" as specified in RFC 7641 [4], the MSGin5G Server shall:

a)	if the message topic does not exist, create the message topic;

b)	if the Originating UE Service ID is not in the list of the subscribers of the message topic, add the Originating UE Service ID to the list of the subscribers of the topic, and record its expiration time if exists;

c)	if an entry with a matching the Originating UE Service ID is already present in the list of the subscribers of the message topic, update the expiration time of the subscription of this UE;

d)	send a CoAP Notifications with a 2.05 (Content) response code to the MSGin5G Client; and

The MSGin5G Server shall remove the Originating UE Service ID from list of the subscribers of the message topic when the expiration time reached;

## 6.7	Usage of Network Capabilities

This clause covers the procedures in MSGin5G-1.

### 6.7.1	General

.

### 6.7.2	UE reachability status monitoring

#### 6.7.2.1	Procedure at MSGin5G Client

#### 6.7.2.2	Procedure at MSGin5G Server

### 6.7.3	MSGin5G device triggering

#### 6.7.3.1	Procedure at MSGin5G Client

#### 6.7.3.2	Procedure at MSGin5G Server

## 6.8	Usage of SEAL

This clause covers the procedures in MSGin5G-1.

### 6.8.1	General

.

### 6.8.2	Configuration management service

#### 6.8.2.1	Procedure at MSGin5G Client

#### 6.8.2.2	Procedure at MSGin5G Server

### 6.8.3	Group management service

#### 6.8.3.1	Procedure at MSGin5G Client

#### 6.8.3.2	Procedure at MSGin5G Server

# 7	Coding

## 7.1	General

## 7.2	MSGin5G UE Configuration data

### 7.2.1	General

This clause specified the extension of the SEAL UE configuration document as defined in 3GPP TS 24.546 [6].

### 7.2.2	Application unique ID

The AUID shall be set to the unique service identifier of MSGin5G service as specified in 3GPP TS 23.554 [2]

### 7.2.3	Structure

The MSGin5G UE configuration document structure is described in clause 7.2 of 3GPP TS 24.546 [6] with the MSGin5G specific clarifications specified in this clause.

The <on-network> element of the <seal-UE-configuration> element specified in clause 7.2 of 3GPP TS 24.546 [6]:

a)	shall include a <MSGin5G-Server-address> element;

b)	shall include a <MSGin5G-UE-Service-id> element; and

c)	may include a <Segment-size> element.

### 7.2.4	XML schema

#### 7.2.4.1	General

The MSGin5G UE configuration document is composed according the XML schema described in the clause 7.2 of 3GPP TS 24.546 [6], and extended with extensions from the XML schema defined in clause 7.2.4.2.

#### 7.2.4.2	XML schema for MSGin5G specific extensions

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema

xmlns="urn:3gpp:ns:seal:MSGin5GUEConfig:1.0"

targetNamespace="urn:3gpp:ns:seal: MSGin5GUEConfig:1.0"

xmlns:xs="http://www.w3.org/2001/XMLSchema"

xmlns: msgin5guec="urn:3gpp:ns:seal: MSGin5GUEConfig:1.0"

elementFormDefault="qualified"

attributeFormDefault="unqualified">

<!—MSGin5G specific "on-network" child elements -->

<xs:element name="MSGin5G-Server-address" type="xs:string"/>

<xs:element name="MSGin5G-UE-Service-id" type="xs:string"/>

<xs:element name="Segment-size" type="xs:unsignedInt"/>

</xs:schema>

### 7.2.5	Data semantics

The <VAL-UE-id> element in <seal-UE-configuration> element is MSGin5G UE ID as specified in TS 23.554 [2].

The <VAL-Service-id> element in <seal-UE-configuration> element is MSGin5G service ID.

The <MSGin5G-Server-address> element in <on-network> element of <seal-UE-configuration> element is the address information of the initial MSGin5G Server serving the MSGin5G Client.

The <MSGin5G-UE-Service-id> element in <on-network> element of <seal-UE-configuration> element is the MSGin5G UE Service ID as specified in TS 23.554 [2].

The <Segment-size> element in <on-network> element of <seal-UE-configuration> element is the segment size used for the MSGin5G UE to do message segmentation, the payload size of every segmented message will not exceed the segment size.

### 7.2.6	MIME types

The MIME type for the MSGin5G UE configuration document shall use the MIME type as specified in the clause 7.2.6 of 3GPP TS 24.546 [6].

## 7.3	MSGin5G message structure

Editor's note: whether more clauses are needed to be added is FFS based on the protocol selection.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-10 | CT1#132e | C1-216109 |  |  |  | Draft skeleton provided by the rapporteur. | 0.0.0 |
| 2021-10 | CT1#132e |  |  |  |  | Implementing the following p-CR agreed by CT1: C1-215739, C1-215873, C1-215874, C1-216174, C1-216177, C1-216180 Editorial change from the rapporteur. Correction from the rapporteur. | 0.1.0 |
| 2021-11 | CT1#133e |  |  |  |  | Implementing the following p-CR agreed by CT1: C1-217092, C1-217293, C1-217294, C1-217295, C1-217296, C1-217330, C1-217331, C1-217332, C1-217334, C1-217335, C1-217338, C1-217339 Editorial change from the rapporteur. Correction from the rapporteur. | 0.2.0 |
| 2021-12 |  |  |  |  |  | Editorial change from the rapporteur. Correction from the rapporteur. | 0.2.1 |