| 3GPP TS 24.550 V0.2.0 (2025-02) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Digital asset, Spatial mapping and Spatial anchors server - Service Enabler Architecture Layer for Verticals (SEAL); Protocol specification; (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	8

4	Overview	8

5	Spatial Anchor and Map Services	8

5.1	Introduction	8

5.2	Spatial Anchor Services	8

5.2.1	SS_SAnManagement Service	8

5.2.1.1	Service Description	8

5.2.1.2	Service Operations	8

5.2.1.2.1	Introduction	8

5.2.1.2.2	SS_SAnManagement_Create	9

5.2.1.2.2.1	General	9

5.2.1.2.2.2	SAn client creating spatial anchor on SAn server using SS_SAnManagement_Create operation	9

5.2.1.2.3	SS_SAnManagement_Update	9

5.2.1.2.3.1	General	9

5.2.1.2.3.2	SAn client updating spatial anchor on SAn server	9

5.2.1.2.4	SS_SAnManagement_Delete	10

5.2.1.2.4.1	General	10

5.2.1.2.4.2	SAn client deleting spatial anchor on SAn server	10

5.2.2	SS_SAnDiscovery Service	10

5.2.2.1	Service Description	10

5.2.2.2	Service Operations	10

5.2.2.2.1	Introduction	10

5.2.2.2.2	SS_SAnDiscovery_Request	11

5.2.2.2.2.1	General	11

5.2.2.2.2.2	SAn client discovering spatial anchors on SAn server	11

5.3	Spatial Map Services	11

5.3.1	SS_SmManagement Service	11

5.3.1.1	Service Description	11

5.3.1.2	Service Operations	11

5.3.1.2.1	Introduction	11

5.3.1.2.2	SS_SmManagement_Create	12

5.3.1.2.2.1	General	12

5.3.1.2.2.2	SM client creating spatial map on SM server using SS_SmManagement_Create operation	12

5.3.1.2.3	SS_SmManagement_Update	12

5.3.1.2.3.1	General	12

5.3.1.2.3.2	SM client updating spatial map on SM server	12

5.3.1.2.4	SS_SmManagement_Delete	13

5.3.1.2.4.1	General	13

5.3.1.2.4.2	SM client deleting spatial map on SM server	13

5.3.2	SS_SmDiscovery Service	14

5.3.2.1	Service Description	14

5.3.2.2	Service Operations	14

5.3.2.2.1	Introduction	14

5.3.3.2.2	SS_SmDiscovery_Request	14

5.3.3.2.2.1	General	14

5.3.3.2.2.2	SM client discovering spatial map on SM server	14

6	API Definitions	14

7	Using Common API Framework	15

Annex A (normative):	Spatial Anchor Server OpenAPI specification	15

A.1	General	15

A.2	SS_SAnManagement API	15

Annex B (normative):	Spatial Map Server OpenAPI specification	15

B.1	General	15

B.2	SS_SmManagement API	15

Annex C (informative):	Change history	15

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the protocol aspects and APIs for the metaverse application over the 3GPP network for SAn-UU and SM-UU reference points. The APIs are specified as RESTful APIs except for custom operations wherever required.

The present document is applicable to the user equipment (UE) supporting the digital asset as described in 3GPP TS 23.438 [3], spatial mapping and spatial anchor client functionalities as described in 3GPP TS 23.437 [2], to the application server supporting the digital asset as described in 3GPP TS 23.438 [3], spatial mapping and spatial anchor server functionalities as described in 3GPP TS 23.437 [2] and to the application server supporting the vertical application server (VAL server) functionality as defined in specific vertical application service (VAL service) specification.

NOTE:	The specification of the VAL server for a specific VAL service is out of scope of the present document.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.437: "Service Enabler Architecture Layer for Verticals (SEAL); Spatial map and Spatial anchors; Stage 2".

[3]	3GPP TS 23.438: "Service Enabler Architecture Layer for Verticals (SEAL); Digital assets Stage 2".

[4]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

[5]	3GPP TS 24.545: "Location Management - Service Enabler Architecture Layer for Verticals (SEAL); Protocol specification".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

SAn-C	Spatial Anchor Client

SAn-S	Spatial Anchor Server

SEAL	Service Enabler Architecture Layer for verticals

SM-C	Spatial Map Client

SM-S	Spatial Map Server

VAL	Vertical Application Layer

# 4	Overview

Editor's note: This clause will provide description of spatial anchor and spatial map server- SEAL service from stage 3 perspective.

# 5	Spatial Anchor and Map Services

## 5.1	Introduction

Table 5.1-x summarizes the corresponding APIs defined for this specification.

Table 5.1-x: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | API Name | Annex |
| --- | --- | --- | --- | --- | --- |
| <service name> | <ref clause> | <short description as included in the OpenAPI file> | <file name> | <apiName in the URI> | <ref Annex> |

NOTE:	When 3GPP TS 29.122 [2] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 5, the service producer (i.e. <NF or Entity, e.g. UAE Server>) takes the role of the SCEF and the service consumer (e.g. <examples of service consumers>) takes the role of the SCS/AS.

## 5.2	Spatial Anchor Services

### 5.2.1	SS_SAnManagement Service

#### 5.2.1.1	Service Description

The SS_SAnManagement API, as defined in 3GPP TS 23.437 [2], allows the SAn client via SSAn-Uu interface to create, update the spatial anchor at a given SAn server.

#### 5.2.1.2	Service Operations

##### 5.2.1.2.1	Introduction

The service operation defined for SS_SAnManagement API is shown in the table 5.2.1.2.1-1.

Table 5.2.1.2.1-1: Operations of the SS_SAnManagement API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| SS_SAnManagement_Create | This service operation is used by the SAn client to create the spatial anchor on SAn server. | SAn client, VAL Server |
| SS_SAnManagement_Update | This service operation is used by the SAn client to update the spatial anchor on SAn server. | SAn client |
| SS_SAnManagement_Delete | This service operation is used by the SAn client to delete the spatial anchor resource on SAn server. | SAn client |

##### 5.2.1.2.2	SS_SAnManagement_Create

###### 5.2.1.2.2.1	General

This service operation is used by SAn client to create the spatial anchor at a given SAn server.

###### 5.2.1.2.2.2	SAn client creating spatial anchor on SAn server using SS_SAnManagement_Create operation

Upon receiving the request from the VAL client for the creation of spatial anchor(s), the SAn client shall send an HTTP POST message to the SAn server as specified in clause "TBD". The body of the HTTP POST message shall include the data structure as specified in clause "TBD".

Upon reception of the HTTP POST message from the SAn client, the SAn server shall:

a)	process the spatial anchor create request;

b)	the SAn server verifies and checks if the SAn client is authorized to create the spatial anchor; and

c)	if the requestor is authorized, the SAn server shall perform the creation of the spatial anchor(s). If the create operation:

1)	is successful, then the SAn server shall further generate a globally unique spatial anchor identifier for each of the newly created spatial anchor. The SAn server shall send the POST response with HTTP "200 OK" status code and the POST response body including the data structure as described in the clause "TBD", which includes the spatial anchor identifier(s) of newly created spatial anchor(s).

NOTE:	Associating the spatial anchor identifier with the VAL service information and requestor identifier is up to implementation.

2)	fails, then the SAn server shall send the POST response set with the appropriate HTTP status code indicating the "failure" and the data structure as described in the clause "TBD".

Editor’s note: The reference to clauses under "TBD" to be resolved with correct reference number.

##### 5.2.1.2.3	SS_SAnManagement_Update

###### 5.2.1.2.3.1	General

This service operation is used by SAn client to update the spatial anchor resource at a given SAn server.

###### 5.2.1.2.3.2	SAn client updating spatial anchor on SAn server

Upon receiving the request from the VAL client to update the spatial anchor resource, the SAn client shall send an HTTP PATCH request (for partial update) or HTTP PUT request (for full replacement) to the SAn server on the resource URI identifying the spatial anchor resource with the spatial anchor information in request message as specified in clause "TBD".

Upon reception of the HTTP PATCH or PUT request from the SAn client, the SAn server shall:

a)	validate if the SAn client is authorized to update the spatial anchor; if the SAn client is unauthorized, the HTTP "403 Forbidden" response is sent; or

b)	if the SAn client is authorized, then SAn server shall check for the spatial anchor resources. If the matching spatial anchor resource is;

1)		found, the SAn server shall;

i)	update the attribute(s) of the matching spatial anchor resource with the received spatial anchor information for the case of HTTP PATCH request; or

Editor’s note: The attribute names provided in the request are FFS.

ii)	replace the matching spatial anchor resource with the received spatial anchor information for the case of HTTP PUT request; and

iii)	the SAn server shall send the HTTP "204 No Content" response to the SAn client; and

2)	not found, return the HTTP "404 Not Found" response to the SAn client.

Editor’s note: The reference to clauses under "TBD" to be resolved with correct reference number.

##### 5.2.1.2.4	SS_SAnManagement_Delete

###### 5.2.1.2.4.1	General

This service operation is used by SAn client to delete the spatial anchor resource at a given SAn server.

###### 5.2.1.2.4.2	SAn client deleting spatial anchor on SAn server

Upon receiving the request from the VAL client to delete the spatial anchor resource, the SAn client shall send an HTTP DELETE request to the SAn server on the resource URI identifying the spatial anchor resource specified in clause "TBD".

Upon reception of the HTTP DELETE request from the SAn client, the SAn server shall:

a)	validate if the SAn client is authorized to delete the spatial anchor; if the SAn client is unauthorized, the HTTP "403 Forbidden" response is sent; or

b)	if the SAn client is authorized, then SAn server shall check for the spatial anchor resources. If the matching spatial anchor resource is;

1)	not found, return "404 Not Found" response to the SAn client; or

2)	found, delete the identified spatial anchor resource and shall send the HTTP "204 No Content" response  to the SAn client.

Editor’s note: The reference to clauses under "TBD" to be resolved with correct reference number.

### 5.2.2	SS_SAnDiscovery Service

#### 5.2.2.1	Service Description

The SS_SAnDiscovery API, as defined in 3GPP TS 23.437 [2], allows the SAn client via SAn-Uu interface to discover the spatial anchor at a given SAn server.

#### 5.2.2.2	Service Operations

##### 5.2.2.2.1	Introduction

The service operation defined for SS_SAnDiscovery API is shown in the table 5.2.2.2.1-1.

Table 5.2.2.2.1-1: Operations of the SS_SAnDiscovery API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| SS_SAnDiscovery_Request | This service operation is used by the SAn client to discover the spatial anchors on SAn server. | SAn client |

##### 5.2.2.2.2	SS_SAnDiscovery_Request

###### 5.2.2.2.2.1	General

This service operation is used by SAn client to discover the spatial anchors at a given SAn server.

###### 5.2.2.2.2.2	SAn client discovering spatial anchors on SAn server

Upon receiving the request from the VAL client to discover the spatial anchors, the SAn client shall send an HTTP POST request (custom operation) to the SAn server.

Upon reception of the HTTP POST request from the SAn client, the SAn server shall:

a)	validate if the SAn client is authorized to perform the spatial anchor discovery; if the SAn client is unauthorized, the HTTP "403 Forbidden" response is sent; or

b)	if the SAn client is authorized, identify the spatial anchors:

1)	that are authorized to be accessed by the SAn client; and

2)	matching the given discovery filters like anchor identity, service identity, area of interest as described in the clause "TBD". If the area of interest is not provided by the SAn client, the SAn server may decide to fetch current location of the SAn client from NEF as described in 3GPP TS 29.522 [4] or by invoking the SEAL Location Management service as described in 3GPP TS 24.545 [5]; and

NOTE:	How the SAn server identifies the spatial anchor(s) based on the provided discovery filters and the VAL UE location of the SAn client is implementation specific.

c)	if the matching spatial anchor resources are:

1)	found, send the HTTP "200 OK" response with the spatial anchor list in response body as described in the clause "TBD" to the SAn client; or

2)	not found, return HTTP "404 Not Found" response to the SAn client.

Upon receiving an HTTP "200 OK" response, the SAn client shall parse the spatial anchor list received in response body and notify the VAL client.

Editor’s note: The reference to clauses under "TBD" to be resolved with correct reference number.

## 5.3	Spatial Map Services

### 5.3.1	SS_SmManagement Service

#### 5.3.1.1	Service Description

The SS_SmManagement API, as defined in 3GPP TS 23.437 [2], allows the SM client via SSAn-Uu interface to create, update the spatial map at a given SM server.

#### 5.3.1.2	Service Operations

##### 5.3.1.2.1	Introduction

The service operation defined for SS_SmManagement API is shown in the table 5.3.1.2.1-1.

Table 5.3.1.2.1-1: Operations of the SS_SmManagement API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| SS_SmManagement_Create | This service operation is used by the SM client to create the spatial map on SM server. | SM client, VAL Server |
| SS_SmManagement_Update | This service operation is used by the SM client to update the spatial map on SM server. | SM client |
| SS_SmManagement_Delete | This service operation is used by the SM client to delete the spatial map on SM server. | SM client |

##### 5.3.1.2.2	SS_SmManagement_Create

###### 5.3.1.2.2.1	General

This service operation is used by SM client to create the spatial map at a given SM server.

###### 5.3.1.2.2.2	SM client creating spatial map on SM server using SS_SmManagement_Create operation

Upon receiving the request from the VAL client for the creation of spatial map, the SM client shall send an HTTP POST message to the SM server as specified in clause "TBD". The body of the HTTP POST message shall include the data structure as specified in clause "TBD".

Upon reception of the HTTP POST message from the SM client, the SM server shall:

a)	process the spatial map create request;

b)	the SM server verifies and checks if the SM client is authorized to create the spatial map; and

c)	if the requestor is authorized, the SM server shall perform the creation of the spatial map. If the create operation:

1)	is successful, then the SM server shall further generate a globally unique spatial map identifier for the newly created spatial map and the layer identifier for the corresponding layers of the map. The SM server shall send the POST response with HTTP "200 OK" status code and the POST response body including the data structure as described in the clause "TBD", which includes the spatial map identifier(s) of newly created spatial map(s).

NOTE:	Associating the spatial map identifier and its associated layer identifiers with the VAL service information and the requestor identifier is up to implementation.

2) 	is in progress, then the SM server shall further generate a globally unique spatial map identifier for the newly created spatial map and shall send the map identifier in the POST response with HTTP "202 Accepted" status code. When the request is processed the SM server shall send a notification to the SM client as described in the clause "TBD".

3)	fails, then the SM server shall send the POST response set with the appropriate HTTP status code indicating the "failure" and the data structure as described in the clause "TBD".

Editor’s note: The reference to clauses under "TBD" to be resolved with correct reference number.

##### 5.3.1.2.3	SS_SmManagement_Update

###### 5.3.1.2.3.1	General

This service operation is used by SM client to update the spatial map resource at a given SM server.

###### 5.3.1.2.3.2	SM client updating spatial map on SM server

Upon receiving the request from the VAL client to update the spatial map resource, the SM client shall send an HTTP PATCH request (for partial update) or HTTP PUT request (for full replacement) to the SM server on the resource URI identifying the spatial map resource with the spatial map information in request message as specified in clause "TBD".

Upon reception of the HTTP PATCH or PUT request from the SM client, the SM server shall:

a)	validate if the SM client is authorized to update the spatial map; if the SM client is unauthorized, the HTTP "403 Forbidden" response is sent; or

b)	if the SM client is authorized, then SM server shall check for the spatial map resources. If the matching spatial map resource is;

1)		found, the SM server shall;

i)	update the attributes(e.g. access control rules, spatial map layering information, spatial map coverage area. augmented layer information etc.) of the matching spatial map resource with the received spatial map information for the case of HTTP PATCH request; or

Editor’s note: The attribute names provided in the request are FFS.

ii)	replace the matching spatial map resource with the received spatial map information for the case of HTTP PUT request; and

iii)	if the update operation:

A)	is successful, then the SM server shall send the HTTP "204 No Content" response to the SM client; and

B)	is in progress, then the SM server shall send the HTTP "202 Accepted" response to the SM client. When the request is processed the SM server shall send a notification to the SM client as described in the clause "TBD"; and

2)	not found, return HTTP "404 Not Found" response to the SM client.

Editor’s note: The reference to clauses under "TBD" to be resolved with correct reference number.

##### 5.3.1.2.4	SS_SmManagement_Delete

###### 5.3.1.2.4.1	General

This service operation is used by SM client to delete the spatial map resource at a given SM server.

###### 5.3.1.2.4.2	SM client deleting spatial map on SM server

Upon receiving the request from the VAL client to delete the spatial map resource, the SM client shall send an HTTP DELETE request to the SM server on the resource URI identifying the spatial map resource as specified in clause "TBD".

Upon reception of the HTTP DELETE request from the SM client, the SM server shall:

a)	validate if the SM client is authorized to delete the spatial map; if the SM client is unauthorized, the HTTP "403 Forbidden" response is sent; or

b)	if the SM client is authorized, then SM server shall check for the spatial map resources. If the matching spatial map resource is;

1)	not found, return the HTTP "404 Not Found" response to the SM client; or

2)	found, delete the identified spatial map resource and shall send the HTTP "204 No Content" response to the SM client.

Editor’s note: The reference to clauses under "TBD" to be resolved with correct reference number.

### 5.3.2	SS_SmDiscovery Service

#### 5.3.2.1	Service Description

The SS_SmDiscovery API, as defined in 3GPP TS 23.437 [2], allows the SM client via SM-Uu interface to discover the spatial map at a given SM server.

#### 5.3.2.2	Service Operations

##### 5.3.2.2.1	Introduction

The service operation defined for SS_SmDiscovery API is shown in the table 5.3.2.2.1-1.

Table 5.3.2.2.1-1: Operations of the SS_SmDiscovery API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| SS_SmDiscovery_Request | This service operation is used by the SM client to discover the spatial map on SM server. | SM client |

##### 5.3.3.2.2	SS_SmDiscovery_Request

###### 5.3.3.2.2.1	General

This service operation is used by SM client to discover the spatial maps at a given SM server.

###### 5.3.3.2.2.2	SM client discovering spatial map on SM server

Upon receiving the request from the VAL client to discover the spatial maps, the SM client shall send an HTTP POST (custom operation) request to the SM server and the spatial map discovery filters in request message as specified in clause "TBD".

Upon reception of the HTTP POST request from the SM client, the SM server shall:

a)	validate if the SM client is authorized to perform the spatial map discovery; if the SM client is unauthorized, the HTTP "403 Forbidden" response is sent; or

b)	if the SM client is authorized, identify the spatial maps that are authorized to be accessed by the SM client and matching the given application service identity and spatial map filters like area of interest, location information, spatial map layers as described in the clause "TBD"; and

NOTE:	How the SM server identifies the spatial map(s) based on the provided map discovery filters and the VAL UE location of the SM client is implementation specific.

c)	if the matching spatial map resources are:

1)	found, send the HTTP "200 OK" response with the spatial map ID list in response body as described in the clause "TBD" to the SM client; or

2)	not found, return HTTP "404 Not Found" response to the SM client.

Upon receiving an HTTP "200 OK" response, the SM client shall parse the spatial map ID list received in response body and notify the VAL client.

Editor’s note: The reference to clauses under "TBD" to be resolved with correct reference number.

# 6	API Definitions

Editor’s note: This clause will describe the spatial anchor and map API definitions based on 3GPP TS 23.437 [2].

# 7	Using Common API Framework

Editor’s note: This clause will describe the CAPIF usage for spatial anchor and map services.

Annex A (normative):
Spatial Anchor Server OpenAPI specification

# A.1	General

# A.2	SS_SAnManagement API

Editor’s note: This clause will define the Open API for spatial anchor based on 3GPP TS 23.437 [2].

Annex B (normative):
Spatial Map Server OpenAPI specification

# B.1	General

# B.2	SS_SmManagement API

Editor’s note: This clause will define the Open API for spatial map based on 3GPP TS 23.437 [2].

Annex C (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-12 | CT#106 |  |  |  |  | Based on latest v0.1.0 of 24.437 as agreed in CT1#152, with TS number change approval from CT#106. | 0.1.0 |
| 2025-02 | CT1#153 |  |  |  |  | Incorporates agreed pCRs C1-250988, C1-250989, C1-251061, C1-251062, C1-251063, C1-251064 | 0.2.0 |