| 3GPP TS 24.572 0.3.0 (2023-10) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; 5G System (5GS); User Plane Location Services (LCS) Protocols And Procedures; Stage 3; (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Abbreviations	9

4	General	9

4.1	Overview	9

4.2	LCS-UP positioning management	9

4.2.1	General	9

4.2.2	PDU session management	9

4.2.3	User plane connection management	10

4.3	Security	10

5	Co-existence of user plane location solutions	10

5.1	General	10

5.2	User plane location solution selection	11

6	User plane positioning connection management	11

6.1	General	11

6.2	UPP-CM procedures	11

6.2.1	Overview	11

6.2.2	Network initiated UPP-CM procedures	11

6.2.2.1	User Plane Connection Information Provisioning	11

6.2.2.1.1	General	11

6.2.3	UE initiated UPP-CM procedures	12

6.2.3.1	UE Originated User Plane Connection Establishment Request	12

6.2.3.1.1	General	12

6.3	UPP-CM messages and coding	13

6.3.1	Messages for UPP-CM	13

6.4	UPP-CM information elements coding	13

6.4.1	Overview	13

6.4.2	UPP-CM information elements	13

7	Elementary procedures for LCS-UPP	14

7.1	Overview	14

7.2	LCS-UPP message transport	14

7.2.1	LCS-UPP message transport in IPv4, IPv6 or IPv4v6 PDU session	14

7.3	LCS-UPP procedures	14

7.3.1	General	14

7.3.2	Uplink LCS-UP transport procedure	15

7.3.2.1	General	15

7.3.2.2	Uplink LCS-UP transport procedure initiation by UE	15

7.3.2.3	Uplink LCS-UP transport procedure accepted by LMF	15

7.3.2.4	Uplink LCS-UP transport procedure not accepted by LMF	15

7.3.2.5	Abnormal cases in the UE	16

7.3.2.5	Abnormal cases on the network side	16

7.3.3	Downlink LCS-UP transport procedure	16

7.3.3.1	General	16

7.3.3.2	Downlink LCS-UPP transport procedure initiation by LMF	16

7.3.3.3	Downlink LCS-UP transport of messages accepted by the UE	16

8	Handling of unknown, unforeseen and erroneous protocol data	17

8.1	General	17

8.2	Message too short or too long	17

8.3	Unknown or unforeseen procedure transaction identity or PDU Session identity	17

8.4	Unknown or unforeseen message type	17

8.5	Non-semantical mandatory information element errors	17

8.6	Unknown and unforeseen IEs in the non-imperative message part	17

8.7	Non-imperative message part errors	17

8.8	Messages with semantically incorrect contents	17

9	Message functional definitions and contents	17

9.1	Overview	17

9.2	LCS-UPP messages	17

9.2.1	UL LCS-UP transport	17

9.2.1.1	Message definition	17

9.2.2	DL LCS-UP transport	18

9.2.2.1	Message definition	18

10	Information elements coding	18

10.1	Overview	18

10.2	LCS-UPP information elements	18

10.2.1	LCS-UP payload	18

10.2.2	LCS-UP payload type	19

10.2.3	Message type	20

11	List of system parameters	20

11.1	General	20

11.2	Timers of LCS-UPP	20

Annex <A> (informative): Change history	21

Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document specifies the user plane protocol to support the Location Services in the 5G System (5GS) as specified in 3GPP TS 23.273 [2] for user plane positioning between the UE and the LMF.

The present document also specifies the LCS user plane connection management procedure to support the user plane connection between the UE and the LMF.

The present document also specifies the user plane protocol to support location event reporting over user plane connection between the UE and the LCS client or the AF.

The present document also defines the message format, message contents, error handling and system parameters applied by the Location Services User plane protocol for supporting Location Services in 5GS.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[3]	3GPP TS 24.571: "5G System (5GS) Control plane Location Services (LCS) procedures; Stage 3".

[4]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[5]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[6]	3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[7]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General aspects".

[8]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[9]	3GPP TS 24.526: "UE policies for 5G System (5GS); Stage 3".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

FQDN	Fully Qualified Domain Name

LCS	LoCation Service

LCS-UP	Location Services User Plane

LCS-UPP	Location Services User Plane Protocol

LMF	Location Management Function

LPP	LTE Positioning Protocol

SUPL	Secure User Plane Location

TLS	Transport Layer Security

UPP-CM	User Plane Positioning Connection Management

4	General

Editor's note: This clause will provide description of User plane location service from stage 3 perspective.

4.1	Overview

4.2	LCS-UP positioning management

4.2.1	General

In order to ensure the transfer of LCS-SS messages and LPP messages via the user plane, the UE and the LMF shall perform the LCS user plane positioning management including:

a)	PDU session management (see subclause 4.2.2);

b)	User plane connection management (see subclause 4.2.3).

4.2.2	PDU session management

A PDU session for the user plane positioning between the UE and the network is a prerequisite for user plane connection between the UE and the LMF for LCS-UPP.

The HPLMN may provide the UE with the URSP rules for the user plane positioning as defined in 3GPP TS 24.526 [9]. The UE supporting the user plane positioning can establish a PDU session for the user plane positioning based on URSP rules. The URSP rules for the user plane positioning includes:

a)	the traffic descriptor containing the connection capability for user plane positioning; and

b)	the route selection descriptor containing a DNN and an S-NSSAI for the user plane positioning.

During the UE-initiated user plane connection establishment procedure and the network-initiated user plane connection establishment procedure, if there is no established PDU session for the user plane positioning, the UE shall initiate a UE-requested PDU session establishment procedure as specified in 3GPP TS 24.501 [4] to establish a new PDU session with a DNN and an S-NSSAI used for the user plane positioning.

NOTE:	The appropriate QoS parameters of the PDU session for the user plane positioning are up to the operator's determination.

4.2.3	User plane connection management

The user plane connection management is to support the establishment, modification, and release procedures for the user plane connection between the UE and the LMF.

To trigger the UE to establish a user plane connection between the UE and the LMF, the LMF shall provide the user plane connection information in the UPP-CMI container encapsulated in the DL NAS TRANSPORT message as defined in 3GPP TS 24.501 [4]. The user plane connection information includes the address of the LMF.

To obtain the user plane connection information, the UE shall include the user plane positioning request message in the UPP-CMI container encapsulated in the UL NAS TRANSPORT message as defined in 3GPP TS 24.501 [4].

The following messages are specified for the user plane connection management:

a)	Messages for UE-initiated user plane connection establishment procedure;

b)	Messages for network-initiated user plane connection establishment procedure;

c)	Messages for network-initiated user plane connection modification procedure;

d)	Messages for network-initiated user plane connection release procedure.

Editor's note:	It is FFS whether further procedures can be defined.

Editor's note:	The detail of the procedures and messages will be defined in the current specification and it is FFS.

4.3	Security

After a successful PDU session establishment providing user plane connectivity, a secured user plane connection between the UE and the LMF for LCS-UPP is protected using a TLS based mechanism as described in 3GPP TS 33.501 [8] annex Q.2.

5	Co-existence of user plane location solutions

5.1	General

The user plane location solution as described in the present specification, i.e. LCS-UPP, may co-exist with other user plane location solutions such as OMA SUPL. The use of OMA SUPL in a 3GPP network is described in 3GPP TS 38.305 [6] and 3GPP TS 23.271 [5].

For UEs supporting another user plane location solution in addition to LCS-UPP, the following indications and parameters can be used to control which user plane location solution is used for such UEs:

a)	LCS-UPP in NAS 5GMM capability IE (see 3GPP TS 24.501 [4]);

b)	SUPL in NAS 5GMM capability IE (see 3GPP TS 24.501 [4]);

c)	LCS-UPP in NAS 5GS network feature support IE (see 3GPP TS 24.501 [4]); and

d)	SUPL in NAS 5GS network feature support IE (see 3GPP TS 24.501 [4]).

5.2	User plane location solution selection

UEs supporting one or more user plane location solution(s) shall indicate its supported user plane location solution(s) to the network using the LCS-UPP bit, the SUPL bit or both in the 5GMM capability IE during the initial and mobility registration procedure as described in 3GPP TS 24.501 [4].

If the UE supports LCS-UPP, SUPL or both, the network shall indicate support of user plane location solution(s) to the UE using LCS-UPP bit, the SUPL bit or both in the 5GS network feature support IE during the initial and mobility registration procedure as described in 3GPP TS 24.501 [4]. The user plane location solution(s) indicated as supported by the network to the UE is selected based on network supported solutions, UE supported solutions and operator policy.

After the UE receives the indication of the supported user plane location solution(s) by the network, and when needed, a user plane location solution supported by both the UE and the network can be initiated and used.

6	User plane positioning connection management

6.1	General

This clause defines the UPP-CM procedures (clause 6.2) and the format and coding of the messages (clause 6.3).

The UPP-CM messages defined in this clause can be included in the UPP-CMI container of the UL NAS Transport messages and DL NAS Transport messages defined in 3GPP TS 24.501 [4].

6.2	UPP-CM procedures

6.2.1	Overview

Editor's note: This clause will provide overview description of the UPP-CM procedures.

6.2.2	Network initiated UPP-CM procedures

6.2.2.1	User Plane Connection Information Provisioning

6.2.2.1.1	General

The user plane connection information provisioning operations enable the network to provide the user plane connection information to the UE as described in clause 6.18.1 of 3GPP TS 23.273 [2]. The user plane connection information is encapsulated in the UPP-CMI container of the DL NAS TRANSPORT message, and the user plane positioning acknowledge is encapsulated in the UPP-CMI container of the UL NAS TRANSPORT message as defined in 3GPP TS 24.501 [4]. Figure 6.2.2.1.1.1 illustrates an example of the NAS signalling transport for user plane connection information provisioning.

Figure 6.2.2.1.1.1: NAS signalling transport for user plane connection information provisioning

6.2.3	UE initiated UPP-CM procedures

6.2.3.1	UE Originated User Plane Connection Establishment Request

6.2.3.1.1	General

The UE originated user plane connection establishment request operations enable the UE to obtain the user plane connection information from the network as described in clause 6.18.2 of 3GPP TS 23.273 [2]. The user plane positioning request message is encapsulated in the UPP-CMI container of the UL NAS TRANSPORT, and the user plane connection information is encapsulated in the UPP-CMI container of the DL NAS TRANSPORT message as defined in 3GPP TS 24.501 [4]. Figure 6.2.3.1.1.1 illustrates an example of the NAS signalling transport for UE originated user plane connection establishment request.

Figure 6.2.3.1.1.1: NAS signalling transport for UE originated user plane connection establishment request

6.3	UPP-CM messages and coding

6.3.1	Messages for UPP-CM

Editor's note: This clause will provide the definitions of UPP-CM messages.

6.4	UPP-CM information elements coding

6.4.1	Overview

Editor's note: This clause will provide overview description of UPP-CM information elements coding.

6.4.2	UPP-CM information elements

Editor's note: This clause will provide the definitions of UPP-CM information elements.

7	Elementary procedures for LCS-UPP

7.1	Overview

Editor's note: This clause will provide description of LCS-UPP elementary procedures.

The main function of the Location Services User Plane protocol (LCS-UPP) is to support generic transport between the UE and the LMF of messages of positioning related protocols:

a)	LPP messages; and

b)	supplementary services messages.

LCS-UPP procedures are performed between a Location Services User Plane (LCS-UP) entity in a UE and an LCS-UP entity in the LMF.

The following UE-initiated LCS-UPP procedures are specified:

a)	UE-initiated LCS-UP transport.

The following LMF-initiated LCS-UPP procedures are specified:

a)	LMF-initiated LCS-UP transport.

Editor’s note:	Additional LCS-UPP procedures between the LMF and the UE, e.g. for LCS-UP connection management, is FFS

The UE-initiated LCS-UPP procedures and the LMF-initiated LCS-UPP procedures can be performed on the user plane connection of a PDU session via 3GPP access in 5GS as specified in clause 6.3.

LCS-UPP messages are transported in an IP packet according to clause 6.2.1.

LCS-UPP is a standard L3 protocol according to 3GPP TS 24.007 [7], LCS-UPP messages are standard L3 messages according to 3GPP TS 24.007 [7] and error behaviour specified for L3 protocol according to 3GPP TS 24.007 [7] applies for LCS-UPP.

7.2	LCS-UPP message transport

7.2.1	LCS-UPP message transport in IPv4, IPv6 or IPv4v6 PDU session

Editor's note: This clause will provide description of LCS-UPP message transport in IPv4, IPv6 or IPv4v6.

Editor's note: This clause will provide information on, and use of additional protocols below LCS-UPP on the connectivity established by the PDU session.

Editor's note: Additional methods for LCS-UPP message transport is FFS and will be added as separate clauses under 7.2, if needed.

7.3	LCS-UPP procedures

Editor's note: This clause will provide specification of LCS-UPP procedures.

7.3.1	General

The LCS-UPP procedure is used to transport both the LPP messages and supplementary services event report messages via a secured user plane connection between UE and LMF as described in clause 6.18.1 step 7 and clause 6.18.2 step 6 of 3GPP TS 23.273 [2].

7.3.2	Uplink LCS-UP transport procedure

7.3.2.1	General

The purpose of the uplink LCS-UPP transport procedure is to provide a transport of an uplink user plane message containing:

a)	LPP message(s); or

b)	supplementary services event report message(s).

7.3.2.2	Uplink LCS-UP transport procedure initiation by UE

The UE initiates the Uplink LCS-UP transport procedure by sending the UL LCS-UP TRANSPORT message to the LMF, as shown in figure 7.3.2.2.1, the UE:

- shall generate the UL LCS-UP TRANSPORT message according to clause 9.2.1 by setting:

1)	the LCS-UP payload type IE indicating which user plane message is transported;

2)	the LCS-UP payload IE which includes at least one of the following;

i)	LPP message(s); or

ii)	Location supplementary services message; and

- shall send the UL LCS-UP TRANSPORT message to the LMF over the user plane connection used for LCS-UPP.

Figure 7.3.2.2.1: Uplink LCS-UP transport procedure initiation by UE

7.3.2.3	Uplink LCS-UP transport procedure accepted by LMF

Upon reception of a UL LCS-UP TRANSPORT message from the UE, if the LCS-UP payload type IE is set to:

a)	"LTE Positioning Protocol (LPP) message", the LCS-UP entity of the LMF shall forward the contents of the LCS-UP payload IE to the LPP entity of the LMF; or

b)	"Location supplementary services message", the LCS-UP entity of the LMF shall forward the contents of the LCS-UP payload IE to the supplementary services entity of the LMF.

7.3.2.4	Uplink LCS-UP transport procedure not accepted by LMF

Editor's note:	Cases and handling of Uplink LCS-UPP transport procedure not accepted by LMF are FFS.

7.3.2.5	Abnormal cases in the UE

7.3.2.5	Abnormal cases on the network side

7.3.3	Downlink LCS-UP transport procedure

7.3.3.1	General

The purpose of the downlink LCS-UPP transport procedure is to provide a transport of a down link user plane message containing LPP message(s).

7.3.3.2	Downlink LCS-UPP transport procedure initiation by LMF

The LMF initiates the downlink LCS-UP transport procedure by sending the DL LCS-UP TRANSPORT message to the UE, as shown in figure 7.3.3.2.1, the LMF:

a)	shall generate the DL LCS-UP TRANSPORT message according to clause 9.2.2 which includes:

1)	the LCS-UP payload type IE to indicate which user plane message is transported; and

2)	the LCS-UP payload IE which includes at least one of the following;

i)	LPP message(s); or

ii)	Location supplementary message; and

b)	shall send the DL LCS-UP TRANSPORT message to the UE over the user plane connection used for LCS-UPP.

Figure 7.3.3.2.1: Downlink LCS-UP transport procedure initiation by LMF

7.3.3.3	Downlink LCS-UP transport of messages accepted by the UE

Upon reception of a DL LCS-UP TRANSPORT message from the LMF, if the LCS-UP payload type IE is set to:

a)	"LTE Positioning Protocol (LPP) message", the LCS-UP entity of the UE shall forward the contents of the LCS-UP payload IE to the LPP entity of the UE; or

b)	"Location supplementary services message", the LCS-UP entity of the UE shall forward the contents of the LCS-UP payload IE to the supplementary services entity of the UE.

8	Handling of unknown, unforeseen and erroneous protocol data

8.1	General

Editor's note: This clause will specify LCS-UPP protocol error handling.

8.2	Message too short or too long

8.3	Unknown or unforeseen procedure transaction identity or PDU Session identity

8.4	Unknown or unforeseen message type

8.5	Non-semantical mandatory information element errors

8.6	Unknown and unforeseen IEs in the non-imperative message part

8.7	Non-imperative message part errors

8.8	Messages with semantically incorrect contents

9	Message functional definitions and contents

9.1	Overview

9.2	LCS-UPP messages

Editor's note: This clause will provide the definitions of LCS-UPP messages.

9.2.1	UL LCS-UP transport

9.2.1.1	Message definition

The UL LCS-UP TRANSPORT message is sent by the UE to the LMF to transport the LPP message(s) or the location supplementary services message. See table 9.2.1.1.1.

Message type:	UL LCS-UP TRANSPORT

Significance:	dual

Direction:	UE to network

Table 9.2.1.1.1: UL LCS-UP TRANSPORT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | UL LCS-UP TRANSPORT message identity | Message type 10.2.3 | M | V | 1 |
|  | LCS-UP payload type | LCS-UP payload type 10.2.2 | M | V | 1 |
|  | LCS-UP payload | LCS-UP payload 10.2.1 | M | LV-E | 3-65537 |

9.2.2	DL LCS-UP transport

9.2.2.1	Message definition

The DL LCS-UP TRANSPORT message is sent by the LMF to the UE to transport the LPP message(s) or the location supplementary services message(s). See table 9.2.2.1.1.

Message type:	DL LCS-UP TRANSPORT

Significance:	dual

Direction:	Network to UE

Table 9.2.2.1.1: DL LCS-UP TRANSPORT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | DL LCS-UP TRANSPORT message identity | Message type 10.2.3 | M | V | 1 |
|  | LCS-UP payload type | LCS-UP payload type 10.2.2 | M | V | 1 |
|  | LCS-UP payload | LCS-UP payload 10.2.1 | M | LV-E | 3-65537 |

10	Information elements coding

10.1	Overview

Editor's note: This clause will provide overview description of LCS-UPP information elements coding.

10.2	LCS-UPP information elements

Editor's note: This clause will provide the definitions of LCS-UPP information elements.

10.2.1	LCS-UP payload

The purpose of the LCS-UP payload information element is to transport LPP message(s) or supplementary services message.

The LCS-UP payload information element is coded as shown in figure 10.2.1.1, figure 10.2.1.2 and table 10.2.1.1.

The LCS-UP payload information element is a type 6 information element with a minimum length of 3 octets and a maximum length of 65538 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LCS-UP payload IEI |  |  |  |  |  |  |  |  |  | octet 1 |
| Length of LCS-UP payload contents |  |  |  |  |  |  |  |  |  | octet 2 |
|  |  |  |  |  |  |  |  |  |  | octet 3 |
|  |  |  |  |  |  |  |  |  |  | octet 4 |
| LCS-UP payload contents |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | octet n |

Figure 10.2.1.1: LCS-UP payload information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of LPP messages |  |  |  |  |  |  |  |  | octet 4 |  |
| Length of LPP message 1 |  |  |  |  |  |  |  |  | octet 5 octet b |  |
| LPP message 1 |  |  |  |  |  |  |  |  | octet (b+1)* octet c |  |
| Length of LPP message 2 |  |  |  |  |  |  |  |  | octet (c+1)* octet d* |  |
| LPP message 2 |  |  |  |  |  |  |  |  | octet (d+1)* octet e* |  |
| … |  |  |  |  |  |  |  |  | octet (e+1)* … octet f* |  |
| Length of LPP message n |  |  |  |  |  |  |  |  | octet (f+1)* octet j* |  |
| LPP message n |  |  |  |  |  |  |  |  | octet (j+1)* octet h* |  |

Figure 10.2.1.2: LCS-UP payload contents with LCS-UP payload type "LTE Positioning Protocol (LPP) message"

Table 10.2.1.1: LCS-UP payload information element

LCS-UP payload contents (octet 4 to octet n);

If the LCS-UP payload type is set to "LTE Positioning Protocol (LPP) message", the LCS-UP payload contents include one or more LPP message(s). If the LCS-UP payload type is set to "Location supplementary services message", the LCS-UP payload contents include a location supplementary services message.

Editor's note:	The detail coding of the Location supplementary services message (i.e. event report) will be defined in the current specification and it is FFS.

10.2.2	LCS-UP payload type

The purpose of the LCS-UP payload type information element indicates information type included in the LCS-UP payload information element.

The LCS-UP payload type information element is coded as shown in figure 10.2.2.1 and table 10.2.2.1

The LCS-UP payload type information element is a type 1 information element.

| 8 |  | 7 | 6 | 5 | 4 |  | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LCS-UP payload type IEI |  |  |  |  | LCS-UP payload type value |  |  |  |  | octet 1 |  |

Figure 10.2.2.1: LCS-UP payload type information element

Table 10.2.2.1: LCS-UP payload type information element

| LCS-UP payload type value (octet 1) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bits |  |  |  |  |
| 4 | 3 | 2 | 1 |  |
| 0 | 0 | 0 | 1 | LTE Positioning Protocol (LPP) message |
| 0 | 0 | 1 | 0 | Location supplementary services message |
| All other values are spare. |  |  |  |  |

10.2.3	Message type

The Message type IE and its use are defined in 3GPP TS 24.007 [11]. Tables 10.2.3.1define the value part of the message type IE used in the LCS-UPP.

The Message type is a type 3 information element, with the length of 1 octet.

Table 10.2.3.1: Message type

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |  | UL LCS-UP TRANSPORT |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |  | DL LCS-UP TRANSPORT |
| All other values are spare. |  |  |  |  |  |  |  |  |  |

11	List of system parameters

11.1	General

11.2	Timers of LCS-UPP

Editor's note: This clause will contain a table of timers of LCS-UPP– UE side, if any.

Editor's note: This clause will contain a table of timers of LCS-UPP– network side, if any.

Annex <A> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2023-03 | CT1#140 | C1-231046 |  |  |  | Draft skeleton provided by the rapporteur. | 0.0.0 |
| 2023-03 | CT1#140 | C1-231047 |  |  |  | Implementing the following p-CRs agreed by CT1: C1-231047; and editorial changes from the rapporteur. | 0.1.0 |
| 2023-08 | CT1#143 | C1-235982, C1-235982, C1-236548, C1-236450, C1-236451, C1-236454, C1-236566 |  |  |  | Implementing the following p-CRs agreed by CT1: C1-235982, C1-235982, C1-236548, C1-236450, C1-236451, C1-236454, C1-236566; and editorial changes from the rapporteur. | 0.2.0 |
| 2023-10 | CT1#144 | C1-237757, C1-237760, C1-237761, C1-237762, C1-237763, C1-237890, C1-237909, C1-237933, C1-237956, C1-237957, C1-237958 |  |  |  | Implementing the following p-CRs agreed by CT1: C1-237757, C1-237760, C1-237761, C1-237762, C1-237763, C1-237890, C1-237909, C1-237933, C1-237956, C1-237957, C1-237958; and editorial changes from the rapporteur. | 0.3.0 |