| 3GPP TS 24.572 1.0.0 (2023-11) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; 5G System (5GS); User Plane Location Services (LCS) Protocols And Procedures; Stage 3; (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Abbreviations	8

4	General	8

4.1	Overview	8

4.2	LCS-UP positioning management	8

4.2.1	General	8

4.2.2	PDU session management	8

4.2.3	User plane positioning connection management	9

4.3	Security	9

5	Co-existence of user plane location solutions	9

5.1	General	9

5.2	User plane location solution selection	9

6	Elementary procedures for UPP-CM	10

6.1	General	10

6.2	UPP-CM procedures	10

6.2.1	Overview	10

6.2.2	Network initiated UPP-CM procedures	10

6.2.2.1	User Plane Connection Information Provisioning	10

6.2.2.1.1	General	10

6.2.2.2	User plane connection release procedure	11

6.2.2.2.1	General	11

6.2.2.2.2	User plane connection release procedure initiation by LMF	12

6.2.2.2.3	User plane connection release procedure accepted by UE	13

6.2.2.2.4	Abnormal cases on the network	13

6.2.3	UE initiated UPP-CM procedures	13

6.2.3.1	UE Originated User Plane Connection Establishment Request	13

6.2.3.1.1	General	13

7	Elementary procedures for LCS-UPP	14

7.1	Overview	14

7.2	LCS-UPP message transport	15

7.2.1	LCS-UPP message transport in IPv4, IPv6 or IPv4v6 PDU session	15

7.3	LCS-UPP procedures	15

7.3.1	General	15

7.3.2	Uplink LCS-UP transport procedure	15

7.3.2.1	General	15

7.3.2.2	Uplink LCS-UP transport procedure initiation by UE	15

7.3.2.3	Uplink LCS-UP transport procedure accepted by LMF	16

7.3.2.4	Uplink LCS-UP transport procedure not accepted by LMF	16

7.3.2.5	Abnormal cases in the UE	16

7.3.2.5	Abnormal cases on the network side	16

7.3.3	Downlink LCS-UP transport procedure	16

7.3.3.1	General	16

7.3.3.2	Downlink LCS-UP transport procedure initiation by LMF	16

7.3.3.3	Downlink LCS-UP transport of messages accepted by the UE	17

9	Handling of unknown, unforeseen and erroneous protocol data	17

9.1	General	17

9.2	Message too short or too long	18

9.2.1	Message too short	18

9.2.2	Message too long	18

9.3	Unknown or unforeseen message type	18

9.4	Non-semantical mandatory information element errors	18

9.5	Unknown and unforeseen IEs in the non-imperative message part	18

9.5.1	IEIs unknown in the message	18

9.5.2	Out of sequence IEs	19

9.5.3	Repeated IEs	19

9.6	Non-imperative message part errors	19

9.6.1	General	19

9.6.2	Syntactically incorrect optional IEs	19

9.6.3	Conditional IE errors	19

9.7	Messages with semantically incorrect contents	19

10	Message functional definitions and contents	19

10.1	Overview	19

10.2	LCS-UPP messages	20

10.2.1	UL LCS-UP transport	20

10.2.1.1	Message definition	20

10.2.2	DL LCS-UP transport	20

10.2.2.1	Message definition	20

10.3	UPP-CM messages	20

10.3.1	User plane connection establishment command	21

10.3.1.1	Message definition	21

10.3.2	User plane connection establishment complete	21

10.3.2.1	Message definition	21

10.3.3	User plane connection establishment reject	21

10.3.3.1	Message definition	21

10.3.4	User plane connection establishment request	22

10.3.4.1	Message definition	22

10.3.5	User plane connection release command	22

10.3.5.1	Message definition	22

10.3.6	User plane connection release complete	22

10.3.6.1	Message definition	22

11	Information elements coding	23

11.1	Overview	23

11.2	LCS-UPP information elements	23

11.2.1	LCS-UP payload	23

11.2.2	LCS-UP payload type	24

11.2.3	Message type	25

11.3	UPP-CM information elements	25

11.3.1	Overview	25

11.3.2	UPP-CM information elements	25

11.3.3	LMF LCS-UP address	25

12	List of system parameters	26

12.1	General	26

12.2	Timers of LCS-UPP	26

Annex <A> (informative): Change history	27

Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document specifies the user plane protocol to support the Location Services in the 5G System (5GS) as specified in 3GPP TS 23.273 [2] for user plane positioning between the UE and the LMF.

The present document also specifies the LCS user plane positioning connection management procedure to support the user plane connection between the UE and the LMF.

The present document also specifies the user plane protocol to support location event reporting over user plane connection between the UE and the LCS client or the AF.

The present document also defines the message format, message contents, error handling and system parameters applied by the Location Services User plane protocol for supporting Location Services in 5GS.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[3]	3GPP TS 24.571: "5G System (5GS) Control plane Location Services (LCS) procedures; Stage 3".

[4]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[5]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[6]	3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[7]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General aspects".

[8]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[9]	3GPP TS 24.526: "UE policies for 5G System (5GS); Stage 3".

[10]	3GPP TS 23.003: "Numbering, addressing and identification".

[11]	3GPP TS 24.080: "Mobile radio interface layer 3 supplementary services specification; Formats and coding".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

FQDN	Fully Qualified Domain Name

LCS	LoCation Service

LCS-UP	Location Services User Plane

LCS-UPP	Location Services User Plane Protocol

LMF	Location Management Function

LPP	LTE Positioning Protocol

SUPL	Secure User Plane Location

TLS	Transport Layer Security

UPP-CM	User Plane Positioning Connection Management

4	General

4.1	Overview

The user plane location services protocols described in the present document provide signalling connectivity between UE and LMF, LCS client or AF, for 5GS.

Main functions of user plane LCS protocols are:

-	support of management of a secured user plane connection for user plane positioning; and

-	user plane transport procedures to provide a transport of LPP and LCS-SS.

For the support of the above functions, the following procedures are supplied within this specification:

-	elementary procedures for user plane positioning connection management, UPP-CM, in clause 6; and

-	elementary procedures for location services user plane protocol, LCS-UPP, in clause 7.

Security, including ciphering and integrity protection, is provided by lower layers from a user plane location services protocol perspective (see 3GPP TS 24.501 [4] for UPP-CM and clause 4.3 for LCS-UPP).

Co-existence of user plane location solutions is described in clause 5.

The UPP-CM and LCS-UPP for 5GS follow the protocol architecture model for layer 3 as described in 3GPP TS 24.007 [7].

4.2	LCS-UP positioning management

4.2.1	General

In order to ensure the transfer of LCS-SS messages and LPP messages via the user plane, the UE and the LMF shall perform the LCS user plane positioning management including:

a)	PDU session management (see subclause 4.2.2);

b)	user plane positioning connection management (see subclause 4.2.3).

4.2.2	PDU session management

A PDU session for the user plane positioning between the UE and the network is a prerequisite for user plane connection between the UE and the LMF for LCS-UPP.

The HPLMN may provide the UE with the URSP rules for the user plane positioning as defined in 3GPP TS 24.526 [9]. The UE supporting the user plane positioning can establish a PDU session for the user plane positioning based on URSP rules. The URSP rules for the user plane positioning includes:

a)	the traffic descriptor containing the connection capability for user plane positioning; and

b)	the route selection descriptor containing a DNN and an S-NSSAI for the user plane positioning.

During the UE-initiated user plane connection establishment procedure and the network-initiated user plane connection establishment procedure, if there is no established PDU session for the user plane positioning, the UE shall initiate a UE-requested PDU session establishment procedure as specified in 3GPP TS 24.501 [4] to establish a new PDU session with a DNN and an S-NSSAI used for the user plane positioning.

NOTE:	The appropriate QoS parameters of the PDU session for the user plane positioning are up to the operator's determination.

4.2.3	User plane positioning connection management

The user plane positioning connection managementis to support the establishment, modification, and release of the user plane connection between the UE and the LMF.

To trigger the UE to establish a user plane connection between the UE and the LMF, the LMF shall provide the user plane connection information to the UE. The UE can also send the user plane positioning request message to the LMF to obtain the user plane connection information. The user plane connection information includes the address of the LMF.

4.3	Security

After a successful PDU session establishment providing user plane connectivity, a secured user plane connection between the UE and the LMF for LCS-UPP is protected using a TLS based mechanism as described in 3GPP TS 33.501 [8] annex Q.2.

5	Co-existence of user plane location solutions

5.1	General

The user plane location solution as described in the present specification, i.e. LCS-UPP, may co-exist with other user plane location solutions such as OMA SUPL. The use of OMA SUPL in a 3GPP network is described in 3GPP TS 38.305 [6] and 3GPP TS 23.271 [5].

For UEs supporting another user plane location solution in addition to LCS-UPP, the following indications and parameters can be used to control which user plane location solution is used for such UEs:

a)	LCS-UPP in NAS 5GMM capability IE (see 3GPP TS 24.501 [4]);

b)	SUPL in NAS 5GMM capability IE (see 3GPP TS 24.501 [4]);

c)	LCS-UPP in NAS 5GS network feature support IE (see 3GPP TS 24.501 [4]); and

d)	SUPL in NAS 5GS network feature support IE (see 3GPP TS 24.501 [4]).

5.2	User plane location solution selection

UEs supporting one or more user plane location solution(s) shall indicate its supported user plane location solution(s) to the network using the LCS-UPP bit, the SUPL bit or both in the 5GMM capability IE during the initial and mobility registration procedure as described in 3GPP TS 24.501 [4].

If the UE supports LCS-UPP, SUPL or both, the network shall indicate support of user plane location solution(s) to the UE using LCS-UPP bit, the SUPL bit or both in the 5GS network feature support IE during the initial and mobility registration procedure as described in 3GPP TS 24.501 [4]. The user plane location solution(s) indicated as supported by the network to the UE is selected based on network supported solutions, UE supported solutions and operator policy.

After the UE receives the indication of the supported user plane location solution(s) by the network, and when needed, a user plane location solution supported by both the UE and the network can be initiated and used.

6	Elementary procedures for UPP-CM

6.1	General

This clause defines the UPP-CM procedures (clause 6.2). The format and coding of the messages and information elements are specified in clause 10.3 and 11.3 respectively.

The user plane positioning connection management is to support the management of a secure user plane connection between the UE and the LMF.

The UPP-CM messages defined in this clause can be included in the UPP-CMI container of the UL NAS Transport messages and DL NAS Transport messages defined in 3GPP TS 24.501 [4].

6.2	UPP-CM procedures

6.2.1	Overview

User plane positioning connection management (UPP-CM) procedures are performed between an LMF and a UE supporting user plane positioning.

The following network initiated UPP-CM procedure is specified:

a)	user plane connection information provisioning procedure.

The following UE initiated UPP-CM procedure is specified:

a)	user plane connection information request procedure.

Editor's note:	It is FFS whether further procedures can be defined.

6.2.2	Network initiated UPP-CM procedures

6.2.2.1	User Plane Connection Information Provisioning

6.2.2.1.1	General

The user plane connection information provisioning operations enable the network to provide the user plane connection information to the UE as described in clause 6.18.1 of 3GPP TS 23.273 [2]. The user plane connection information is encapsulated in the UPP-CMI container of the DL NAS TRANSPORT message, and the user plane positioning acknowledge is encapsulated in the UPP-CMI container of the UL NAS TRANSPORT message as defined in 3GPP TS 24.501 [4]. Figure 6.2.2.1.1.1 illustrates an example of the NAS signalling transport for user plane connection information provisioning.

Figure 6.2.2.1.1.1: NAS signalling transport for user plane connection information provisioning

6.2.2.2	User plane connection release procedure

6.2.2.2.1	General

The user plane connection release procedure enables the network to release the connection between the UE and the LMF via the control plane as described in clause 6.18.3 of 3GPP TS 23.273 [2]. The user plane connection release command message is encapsulated in the UPP-CMI container of the DL NAS TRANSPORT message, and the user plane connection release complete message is encapsulated in the UPP-CMI container of the UL NAS TRANSPORT message. Figure 6.2.2.2.1.1 illustrates an example of the NAS signalling transport for the user plane connection release procedure.

Figure 6.2.2.2.1.1: NAS signalling transport for user plane connection release procedure

6.2.2.2.2	User plane connection release procedure initiation by LMF

The LMF initiates the user plane connection release procedure by sending the USER PLANE CONNECTION RELEASE COMMAND message to the UE, as shown in figure 6.2.2.2.2.1, the LMF:

a)	shall generate the USER PLANE CONNECTION RELEASE COMMAND message according to subclause 10.3.5,

b)	shall send the USER PLANE CONNECTION RELEASE COMMAND message to the UE; and

c)	shall start a timer T5010 upon sending the USER PLANE CONNECTION RELEASE COMMAND message.

Figure 6.2.2.2.2.1: LMF-initiated user plane connection release procedure

6.2.2.2.3	User plane connection release procedure accepted by UE

Upon receipt of a USER PLANE CONNECTION RELEASE COMMAND message from the LMF, the UE shall stop sending LCS-UPP messages, terminate the TLS connection, create a USER PLANE CONNECTION RELEASE COMPLETE message according to clause 10.3.6 and send it to the LMF, and consider the user plane connection between the UE and the LMF as released.

Upon reception of a USER PLANE CONNECTION RELEASE COMPLETE message from the UE, the LMF shall stop the timer T5010 and shall consider the user plane connection between the UE and the LMF as released.

6.2.2.2.4	Abnormal cases on the network

The following abnormal cases can be identified:

a)	Expiry of the timer T5010

The LMF shall, on the first expiry of the timer T5010, retransmit the USER PLANE CONNECTION RELEASE COMMAND message and shall reset and start timer T5010. This retransmission is repeated up to four times, i.e. on the fifth expiry of timer T5010, the LMF shall abort the procedure and locally release the user plane connection between the UE and the LMF.

6.2.3	UE initiated UPP-CM procedures

6.2.3.1	UE Originated User Plane Connection Establishment Request

6.2.3.1.1	General

The UE originated user plane connection establishment request operations enable the UE to obtain the user plane connection information from the network as described in clause 6.18.2 of 3GPP TS 23.273 [2]. The user plane positioning request message is encapsulated in the UPP-CMI container of the UL NAS TRANSPORT, and the user plane connection information is encapsulated in the UPP-CMI container of the DL NAS TRANSPORT message as defined in 3GPP TS 24.501 [4]. Figure 6.2.3.1.1.1 illustrates an example of the NAS signalling transport for UE originated user plane connection establishment request.

Figure 6.2.3.1.1.1: NAS signalling transport for UE originated user plane connection establishment request

7	Elementary procedures for LCS-UPP

7.1	Overview

The main function of the Location Services User Plane protocol (LCS-UPP) is to support generic transport between the UE and the LMF of messages of positioning related protocols:

a)	LPP messages; and

b)	location supplementary services messages, only including:

1)	messages for EventReport operations (see 3GPP TS 24.080 [11]);

2)	messages for PeriodicTriggeredInvoke operations (see 3GPP TS 24.080 [11]); and

3)	messages for MSCancelDeferredLocation operations (see 3GPP TS 24.080 [11]).

LCS-UPP procedures are performed between a Location Services User Plane (LCS-UP) entity in a UE and an LCS-UP entity in the LMF.

The following UE-initiated LCS-UPP procedures are specified:

a)	uplink LCS-UP transport procedure.

The following LMF-initiated LCS-UPP procedures are specified:

a)	downlink LCS-UP transport procedure.

Editor’s note:	Additional LCS-UPP procedures between the LMF and the UE, e.g. for LCS-UP connection management, is FFS

The uplink LCS-UP transport procedures and the downlink LCS-UP transport procedures can be performed on the user plane connection of a PDU session via 3GPP access in 5GS as specified in clause 7.3.

LCS-UPP messages are transported in an IP packet according to clause 7.2.1.

LCS-UPP is a standard L3 protocol according to 3GPP TS 24.007 [7], LCS-UPP messages are standard L3 messages according to 3GPP TS 24.007 [7] and error behaviour specified for L3 protocol according to 3GPP TS 24.007 [7] applies for LCS-UPP.

7.2	LCS-UPP message transport

7.2.1	LCS-UPP message transport in IPv4, IPv6 or IPv4v6 PDU session

In order to send an LCS-UPP message over a PDU session of IPv4, IPv6 or IPv4v6 PDU session type, the UE shall establish the TLS connection. The UE and the LMF shall use the TLS connection to exchange LCS-UP messages.

7.3	LCS-UPP procedures

7.3.1	General

The LCS-UPP transport procedure is used to transport both the LPP messages and the location supplementary services messages via a secured user plane connection between the UE and the LMF.

7.3.2	Uplink LCS-UP transport procedure

7.3.2.1	General

The purpose of the uplink LCS-UP transport procedure is to provide a transport of an uplink user plane message containing:

a)	LPP message(s); or

b)	location supplementary services message (see clause 7.1).

7.3.2.2	Uplink LCS-UP transport procedure initiation by UE

The UE initiates the Uplink LCS-UP transport procedure by sending the UL LCS-UP TRANSPORT message to the LMF, as shown in figure 7.3.2.2.1, the UE:

- shall generate the UL LCS-UP TRANSPORT message according to clause 10.2.1 by setting:

1)	the LCS-UP payload type IE indicating which user plane message is transported;

2)	the LCS-UP payload IE which includes at least one of the following;

i)	LPP message(s); or

ii)	Location supplementary services message; and

- shall send the UL LCS-UP TRANSPORT message to the LMF over the user plane connection used for LCS-UPP.

Figure 7.3.2.2.1: Uplink LCS-UP transport procedure initiation by UE

7.3.2.3	Uplink LCS-UP transport procedure accepted by LMF

Upon reception of a UL LCS-UP TRANSPORT message from the UE, if the LCS-UP payload type IE is set to:

a)	"LTE Positioning Protocol (LPP) message", the LCS-UP entity of the LMF shall forward the contents of the LCS-UP payload IE to the LPP entity of the LMF; or

b)	"Location supplementary services message", the LCS-UP entity of the LMF shall forward the contents of the LCS-UP payload IE to the supplementary services entity of the LMF.

7.3.2.4	Uplink LCS-UP transport procedure not accepted by LMF

Editor's note:	Cases and handling of Uplink LCS-UPP transport procedure not accepted by LMF are FFS.

7.3.2.5	Abnormal cases in the UE

7.3.2.5	Abnormal cases on the network side

7.3.3	Downlink LCS-UP transport procedure

7.3.3.1	General

The purpose of the downlink LCS-UP transport procedure is to provide a transport of a down link user plane message containing:

a)	LPP message(s); or

b)	location supplementary services message (see clause 7.1).

7.3.3.2	Downlink LCS-UP transport procedure initiation by LMF

The LMF initiates the downlink LCS-UP transport procedure by sending the DL LCS-UP TRANSPORT message to the UE, as shown in figure 7.3.3.2.1, the LMF:

a)	shall generate the DL LCS-UP TRANSPORT message according to clause 10.2.2 which includes:

1)	the LCS-UP payload type IE to indicate which user plane message is transported; and

2)	the LCS-UP payload IE which includes at least one of the following;

i)	LPP message(s); or

ii)	Location supplementary services message; and

b)	shall send the DL LCS-UP TRANSPORT message to the UE over the user plane connection used for LCS-UPP.

Figure 7.3.3.2.1: Downlink LCS-UP transport procedure initiation by LMF

7.3.3.3	Downlink LCS-UP transport of messages accepted by the UE

Upon reception of a DL LCS-UP TRANSPORT message from the LMF, the LCS-UP entity of the UE shall forward the LCS-UP payload type, the contents of the LCS-UP payload IE to the upper layer location services application.

# 8	LCS-UPP procedures for LCS client or AF

If the LCS-UP entity is implemented both in the UE and the LCS client or the AF, LCS-UPP procedures may be performed between a LCS-UP entity in a UE and an LCS-UP entity in the LCS client or the AF in order to support the location supplementary services event report via user plane procedure as described in clause 6.16 of 3GPP TS 23.273 [2].

The UE uses the information of user plane connection provisioned by the network, as specified in clause 5.2.1.3 of 3GPP TS 24.571 [3], to establish secure user plane connection with the LCS client or AF.

NOTE:	How to manage a secure user plane connection with the LCS client or the AF is out of scope of this specification.

The UE initiates the uplink LCS-UP transport procedure as defined in clause 7.3.2 to send the location supplementary services event report to the LCS client or AF, and the LCS client or the AF initiates the downlink LCS-UP transport procedure as defined in clause 7.3.3 to send the location supplementary services acknowledgement of event report to the UE.

9	Handling of unknown, unforeseen and erroneous protocol data

9.1	General

The procedures specified in the present document apply to those LCS-UPP messages and UPP-CM messages which pass the checks described in this clause.

This clause also specifies procedures for the handling of unknown, unforeseen and erroneous protocol data by the receiving entity. These procedures are called "error handling procedures", but in addition to providing recovery mechanisms for error situations they define a compatibility mechanism for future extensions of the protocols.

Detailed error handling procedures in the network are implementation dependent and may vary from PLMN to PLMN. However, when extensions of this protocol are developed, networks are assumed to have the error handling which is indicated in this clause as mandatory ("shall") and that is indicated as strongly recommended ("should").

Also, the error handling of the network is only considered as mandatory or strongly recommended when certain thresholds for errors are not reached during a dedicated connection.

For definition of semantical and syntactical errors see 3GPP TS 24.007 [7], clause 11.4.2.

9.2	Message too short or too long

9.2.1	Message too short

When a message is received that is too short to contain a complete message type information element, that message shall be ignored, cf. 3GPP TS 24.007 [7].

9.2.2	Message too long

The maximum size of a LCS-UPP message or a UPP-CM message is 65535 octets.

9.3	Unknown or unforeseen message type

If the UE receives a LCS-UPP message with message type not defined for the LCS-UPP or not implemented by the receiver, it shall ignore the LCS-UPP message.

NOTE 1:	A message type not defined for the LCS-UPP in the given direction is regarded by the receiver as a message type not defined for the LCS-UPP, see 3GPP TS 24.007 [7].

If the UE receives a message not compatible with the LCS-UPP state, the UE shall ignore the LCS-UPP message.

If the UE receives a UPP-CM message with message type not defined for the UPP-CM or not implemented by the receiver, it shall ignore the UPP-CM message.

NOTE 2:	A message type not defined for the UPP-CM in the given direction is regarded by the receiver as a message type not defined for the UPP-CM, see 3GPP TS 24.007 [7].

If the UE receives a message not compatible with the UPP-CM state, the UE shall ignore the UPP-CM message.

9.4	Non-semantical mandatory information element errors

When on receipt of a message,

a)	an "imperative message part" error; or

b)	a "missing mandatory IE" error

is diagnosed or when a message containing:

a)	a syntactically incorrect mandatory IE;

b)	an IE unknown in the message, but encoded as "comprehension required" (see 3GPP TS 24.007 [7]); or

c)	an out of sequence IE encoded as "comprehension required" (see 3GPP TS 24.007 [7]) is received,

the UE shall ignore the LCS-UPP message or UPP-CM the message.

9.5	Unknown and unforeseen IEs in the non-imperative message part

9.5.1	IEIs unknown in the message

The UE shall ignore all IEs unknown in a message which are not encoded as "comprehension required" (see 3GPP TS 24.007 [7]).

9.5.2	Out of sequence IEs

The UE shall ignore all out of sequence IEs in a message which are not encoded as "comprehension required" (see 3GPP TS 24.007 [7]).

9.5.3	Repeated IEs

If an information element with format T, TV, TLV, or TLV-E is repeated in a message in which repetition of the information element is not specified in clause 10, the UE shall handle only the contents of the information element appearing first and shall ignore all subsequent repetitions of the information element. When repetition of information elements is specified, the UE shall handle only the contents of specified repeated information elements. If the limit on repetition of information elements is exceeded, the UE shall handle the contents of information elements appearing first up to the limit of repetitions and shall ignore all subsequent repetitions of the information element.

9.6	Non-imperative message part errors

9.6.1	General

This category includes:

a)	syntactically incorrect optional IEs; and

b)	conditional IE errors.

9.6.2	Syntactically incorrect optional IEs

The UE shall treat all optional IEs that are syntactically incorrect in a message as not present in the message.

9.6.3	Conditional IE errors

When upon receipt of a LCS-UPP message, the UE diagnoses a "missing conditional IE" error or an "unexpected conditional IE" error, or when it receives a LCS-UPP message containing at least one syntactically incorrect conditional IE, the UE shall ignore the message.

When upon receipt of a UPP-CM message, the UE diagnoses a "missing conditional IE" error or an "unexpected conditional IE" error, or when it receives a UPP-CM message containing at least one syntactically incorrect conditional IE, the UE shall ignore the message.

9.7	Messages with semantically incorrect contents

When a message with semantically incorrect contents is received, the UE shall perform the foreseen reactions of the procedural part of clause 7.3. If, however no such reactions are specified, the UE shall ignore the message.

10	Message functional definitions and contents

10.1	Overview

This clause contains the definition and contents of the messages used in the procedures described in the present document, which include:

a)	LCS-UPP messages as defined in clause 10.2; and

b)	UPP-CM messages as defined in clause 10.3.

10.2	LCS-UPP messages

Editor's note: This clause will provide the definitions of LCS-UPP messages.

10.2.1	UL LCS-UP transport

10.2.1.1	Message definition

The UL LCS-UP TRANSPORT message is sent by the UE to the LMF to transport the LPP message(s) or the location supplementary services message. See table 10.2.1.1.1.

Message type:	UL LCS-UP TRANSPORT

Significance:	dual

Direction:	UE to network

Table 10.2.1.1.1: UL LCS-UP TRANSPORT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | UL LCS-UP TRANSPORT message identity | Message type 11.2.3 | M | V | 1 |
|  | LCS-UP payload type | LCS-UP payload type 11.2.2 | M | V | 1 |
|  | LCS-UP payload | LCS-UP payload 11.2.1 | M | LV-E | 3-65537 |

10.2.2	DL LCS-UP transport

10.2.2.1	Message definition

The DL LCS-UP TRANSPORT message is sent by the LMF to the UE to transport the LPP message(s) or the location supplementary services message(s). See table 10.2.2.1.1.

Message type:	DL LCS-UP TRANSPORT

Significance:	dual

Direction:	Network to UE

Table 10.2.2.1.1: DL LCS-UP TRANSPORT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | DL LCS-UP TRANSPORT message identity | Message type 11.2.3 | M | V | 1 |
|  | LCS-UP payload type | LCS-UP payload type 11.2.2 | M | V | 1 |
|  | LCS-UP payload | LCS-UP payload 11.2.1 | M | LV-E | 3-65537 |

10.3	UPP-CM messages

Editor's note: This clause will provide the definitions of UPP-CM messages.

The following UPP-CM messages are specified:

-	the USER PLANE CONNECTION RELEASE COMMAND message (see subclause 10.3.5); and

-	the USER PLANE CONNECTION RELEASE COMPLETE message (see subclause 10.3.6).

10.3.1	User plane connection establishment command

10.3.1.1	Message definition

The user plane connection establishment command message is sent by the LMF to the UE to transport the user plane connection information and trigger the UE to establish the TLS connection towards the LMF. See table 10.3.1.1.1.

Message type:	User Plane Connection Establishment Command

Significance:	dual

Direction:	Network to UE

Table 10.3.1.1.1: User Plane Connection Establishment Command message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | User Plane Connection Establishment Command identity | Message type 11.2.3 | M | V | 1 |
| 11 | LMF LCS-UP address | LMF LCS-UP address 11.3.3 | O | TLV | 4-257 |

Editor's note: Whether LMF LCS-UP address is mandatory or optional is FFS.

10.3.2	User plane connection establishment complete

10.3.2.1	Message definition

The user plane connection complete message is sent by the UE to the LMF to acknowledge the establishment command received from LMF. See table 10.3.2.1.1.

Message type:	User Plane Connection Establishment Complete

Significance:	dual

Direction:	UE to Network

Table 10.3.2.1.1: User Plane Connection Establishment Complete message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | User Plane Connection Establishment Complete message identity | Message type 11.2.3 | M | V | 1 |

10.3.3	User plane connection establishment reject

10.3.3.1	Message definition

The user plane connection reject message is sent by the UE to the LMF to reject the establishment command received from LMF. See table 10.3.3.1.1.

Message type:	User Plane Connection Establishment Reject

Significance:	dual

Direction:	UE to Network

Table 10.3.3.1.1: User Plane Connection Establishment Reject message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | User Plane Connection Establishment Reject message identity | Message type 11.2.3 | M | V | 1 |

10.3.4	User plane connection establishment request

10.3.4.1	Message definition

The user plane connection establishment request message is sent by the UE to the LMF to establish a user plane connection. See table 10.3.4.1.1.

Message type:	User Plane Connection Establishment Request

Significance:	dual

Direction:	UE to Network

Table 10.3.4.1.1: User Plane Connection Establishment Request message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | User Plane Connection Establishment Request message identity | Message type 11.2.3 | M | V | 1 |

10.3.5	User plane connection release command

10.3.5.1	Message definition

The USER PLANE CONNECTION RELEASE COMMAND message is sent by the LMF to the UE to release the user plane connection between the UE and the LMF. See table 10.3.5.1.1.

Message type:	USER PLANE CONNECTION RELEASE COMMAND

Significance:	dual

Direction:	Network to UE

Table 10.3.5.1.1: USER PLANE CONNECTION RELEASE COMMAND message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | USER PLANE CONNECTION RELEASE COMMAND message identity | Message type 11.2.3 | M | V | 1 |

10.3.6	User plane connection release complete

10.3.6.1	Message definition

The USER PLANE CONNECTION RELEASE COMPLETE message is sent by the LMF to the UE to response the command to release the user plane connection between the UE and the LMF. See table 10.3.6.1.1.

Message type:	USER PLANE CONNECTION RELEASE COMPLETE

Significance:	dual

Direction:	UE to network

Table 10.3.6.1.1: USER PLANE CONNECTION RELEASE COMPLETE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | USER PLANE CONNECTION RELEASE COMPLETE message identity | Message type 11.2.3 | M | V | 1 |

11	Information elements coding

11.1	Overview

Editor's note: This clause will provide overview description of LCS-UPP information elements coding.

11.2	LCS-UPP information elements

11.2.1	LCS-UP payload

The purpose of the LCS-UP payload information element is to transport LPP message(s) or supplementary services message.

The LCS-UP payload information element is coded as shown in figure 11.2.1.1, figure 11.2.1.2 and table 11.2.1.1.

The LCS-UP payload information element is a type 6 information element with a minimum length of 3 octets and a maximum length of 65538 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LCS-UP payload IEI |  |  |  |  |  |  |  |  |  | octet 1 |
| Length of LCS-UP payload contents |  |  |  |  |  |  |  |  |  | octet 2 |
|  |  |  |  |  |  |  |  |  |  | octet 3 |
|  |  |  |  |  |  |  |  |  |  | octet 4 |
| LCS-UP payload contents |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | octet n |

Figure 11.2.1.1: LCS-UP payload information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of LPP messages |  |  |  |  |  |  |  |  | octet 4 |  |
| Length of LPP message 1 |  |  |  |  |  |  |  |  | octet 5 octet b |  |
| LPP message 1 |  |  |  |  |  |  |  |  | octet (b+1)* octet c* |  |
| Length of LPP message 2 |  |  |  |  |  |  |  |  | octet (c+1)* octet d* |  |
| LPP message 2 |  |  |  |  |  |  |  |  | octet (d+1)* octet e* |  |
| … |  |  |  |  |  |  |  |  | octet (e+1)* … octet f* |  |
| Length of LPP message n |  |  |  |  |  |  |  |  | octet (f+1)* octet j* |  |
| LPP message n |  |  |  |  |  |  |  |  | octet (j+1)* octet h* |  |

Figure 11.2.1.2: LCS-UP payload contents with LCS-UP payload type "LTE Positioning Protocol (LPP) message"

Table 11.2.1.1: LCS-UP payload information element

LCS-UP payload contents (octet 4 to octet n);

If the LCS-UP payload type is set to "LTE Positioning Protocol (LPP) message", the LCS-UP payload contents include one or more LPP message(s). If the LCS-UP payload type is set to "Location supplementary services message", the LCS-UP payload contents include a location supplementary services message.

Editor's note:	The detail coding of the Location supplementary services message (i.e. event report) will be defined in the current specification and it is FFS.

11.2.2	LCS-UP payload type

The purpose of the LCS-UP payload type information element indicates information type included in the LCS-UP payload information element.

The LCS-UP payload type information element is coded as shown in figure 11.2.2.1 and table 11.2.2.1

The LCS-UP payload type information element is a type 1 information element.

| 8 |  | 7 | 6 | 5 | 4 |  | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LCS-UP payload type IEI |  |  |  |  | LCS-UP payload type value |  |  |  |  | octet 1 |  |

Figure 11.2.2.1: LCS-UP payload type information element

Table 11.2.2.1: LCS-UP payload type information element

| LCS-UP payload type value (octet 1) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bits |  |  |  |  |
| 4 | 3 | 2 | 1 |  |
| 0 | 0 | 0 | 1 | LTE Positioning Protocol (LPP) message |
| 0 | 0 | 1 | 0 | Location supplementary services message |
| All other values are spare. |  |  |  |  |

11.2.3	Message type

The Message type IE and its use are defined in 3GPP TS 24.007 [11]. Tables 11.2.3.1defines the value part of the message type IE used in the LCS-UPP and tables 11.2.3.2 defines the value part of the message type IE used in the UPP-CM.

The Message type is a type 3 information element, with the length of 1 octet.

Table 11.2.3.1: Message type for LCS-UPP

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 1 | - | - | - | - | - | - |  | LCS-UPP messages |
| 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 |  | UL LCS-UP TRANSPORT |
| 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 |  | DL LCS-UP TRANSPORT |

Table 11.2.3.2: Message type for UPP-CM

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 1 | 1 | - | - | - | - | - | - |  | UPP-CM messages |
| 1 | 1 | 0 | 0 | 0 | 0 | 0 | 1 |  | User Plane Connection Establishment Command |
| 1 | 1 | 0 | 0 | 0 | 0 | 1 | 0 |  | User Plane Connection Establishment Complete |
| 1 | 1 | 0 | 0 | 0 | 0 | 1 | 1 |  | User Plane Connection Establishment Reject |
| 1 | 1 | 0 | 0 | 0 | 1 | 0 | 0 |  | User Plane Connection Establishment Request |
| 1 | 1 | 0 | 0 | 0 | 1 | 0 | 1 |  | User Plane Connection Release Command |
| 1 | 1 | 0 | 0 | 0 | 1 | 1 | 0 |  | User Plane Connection Release Response |

11.3	UPP-CM information elements

11.3.1	Overview

Editor's note: This clause will provide overview description of UPP-CM information elements coding.

11.3.2	UPP-CM information elements

Editor's note: This clause will provide the definitions of UPP-CM information elements.

11.3.3	LMF LCS-UP address

The purpose of the LMF LCS-UP address information element is to carry the user plane positioning address of the LMF.

The LMF LCS-UP address information element is coded as shown in figure 11.3.3.1 and table 11.3.3.1.

The LMF LCS-UP address information element is a type 4 information element with minimum length of 4 octets and maximum length of 257 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LMF LCS-UP address IEI |  |  |  |  |  |  |  | octet 1 |
| LMF LCS-UP address length |  |  |  |  |  |  |  | octet 2 |
| LMF LCS-UP address type |  |  |  |  |  |  |  | octet 3 |
| LMF LCS-UP address |  |  |  |  |  |  |  | octets 4-z |

Figure 11.3.3.1: LMF LCS-UP address information element

Table 11.3.3.1: LMF LCS-UP address information element

| LMF LCS-UP address type (octet 3) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bits |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |  | IPv4 |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |  | IPv6 |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |  | IPv4v6 |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |  | FQDN |
| All other values are spare. If received they shall be ignored. |  |  |  |  |  |  |  |  |  |
| If the LMF LCS-UP address type indicates IPv4, then the LMF LCS-UP address field contains an IPv4 address in octet 4 to octet 7. |  |  |  |  |  |  |  |  |  |
| If the LMF LCS-UP address type indicates IPv6, then the LMF LCS-UP address field contains an IPv6 address in octet 4 to octet 19. |  |  |  |  |  |  |  |  |  |
| If the LMF LCS-UP address type indicates IPv4v6, then the LMF LCS-UP address field contains two IP addresses. The first IP address is an IPv4 address in octet 4 to octet 7. The second IP address is an IPv6 address in octet 8 to octet 23. |  |  |  |  |  |  |  |  |  |
| If the LMF LCS-UP address type indicates FQDN, octet 4 to octet z is encoded as defined in subclause 19.4.2.1 in 3GPP TS 23.003 [10]. |  |  |  |  |  |  |  |  |  |

12	List of system parameters

12.1	General

12.2	Timers of LCS-UPP

Editor's note: This clause will contain a table of timers of LCS-UPP– UE side, if any.

Editor's note: This clause will contain a table of timers of LCS-UPP– network side, if any.

Annex <A> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2023-03 | CT1#140 | C1-231046 |  |  |  | Draft skeleton provided by the rapporteur. | 0.0.0 |
| 2023-03 | CT1#140 | C1-231047 |  |  |  | Implementing the following p-CRs agreed by CT1: C1-231047; and editorial changes from the rapporteur. | 0.1.0 |
| 2023-08 | CT1#143 | C1-235982, C1-235982, C1-236548, C1-236450, C1-236451, C1-236454, C1-236566 |  |  |  | Implementing the following p-CRs agreed by CT1: C1-235982, C1-235982, C1-236548, C1-236450, C1-236451, C1-236454, C1-236566; and editorial changes from the rapporteur. | 0.2.0 |
| 2023-10 | CT1#144 | C1-237757, C1-237760, C1-237761, C1-237762, C1-237763, C1-237890, C1-237909, C1-237933, C1-237956, C1-237957, C1-237958 |  |  |  | Implementing the following p-CRs agreed by CT1: C1-237757, C1-237760, C1-237761, C1-237762, C1-237763, C1-237890, C1-237909, C1-237933, C1-237956, C1-237957, C1-237958; and editorial changes from the rapporteur. | 0.3.0 |
| 23023-11 | CT1#145 | C1-239250, C1-239251. C1-239252. C1-239253. C1-239650. C1-239656. C1-239258, C1-239260, C1-239261, C1-239264, C1-239380, C1-239373, C1-239651, |  |  |  | Implementing the following p-CRs agreed by CT1: C1-239250, C1-239251. C1-239252. C1-239253. C1-239650. C1-239656. C1-239258, C1-239260, C1-239261, C1-239264, C1-239380, C1-239373, C1-239651; and editorial changes from the rapporteur. | 0.4.0 |