| 3GPP TS 24.575 V18.0.0 (2023-03) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; 5G System; Multicast/Broadcast UE pre-configuration Management Object (MO) (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	5

2	References	5

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Abbreviations	6

4	General description	6

5	Multicast/Broadcast UE pre-configuration	7

5.1	General	7

5.2	UE pre-configuration MO structure	7

6	UE pre-configuration MO parameters	7

6.1	General	7

6.2	Node: <X>	7

6.3	<X>/Name	8

6.4	<X>/PLMNList	8

6.5	<X>/PLMNList/<X>/	8

6.6	<X>/PLMNList/<X>/PLMNId	8

6.7	<X>/PLMNList/<X>/TMGIConfiguration	9

6.8	<X>/PLMNList/<X>/TMGIConfiguration/TMGIListForSA	9

6.9	<X>/PLMNList/<X>/TMGIConfiguration/TMGIListForSA/<X>	9

6.10	<X>/PLMNList/<X>/TMGIConfiguration/TMGIListForSA/<X>/TMGI	9

6.11	<X>/PLMNList/<X>/TMGIConfiguration/TMGIListForSA/<X>/USD	10

6.12	<X>/PLMNList/<X>/TMGIConfiguration/TMGIListForService	10

6.13	<X>/PLMNList/<X>/TMGIConfiguration/TMGIListForService/<X>	10

6.14	<X>/PLMNList/<X>/TMGIConfiguration/TMGIListForService/<X>/TMGI	10

6.15	<X>/PLMNList/<X>/TMGIConfiguration/TMGIListForService/<X>/USD	11

6.16	<X>/PLMNList/<X>/RANInfo	11

6.17	<X>/PLMNList/<X>/RANInfo/<X>	11

6.18	<X>/PLMNList/<X>/RANInfo/<X>/NRARFCN	11

6.19	<X>/PLMNList/<X>/PDUInfo	12

6.20	<X>/PLMNList/<X>/PDUInfo/PDUInfoList	12

6.21	<X>/PLMNList/<X>/PDUInfo/PDUInfoList/<X>	12

6.22	<X>/PLMNList/<X>/PDUInfo/PDUInfoList/<X>/DNN	12

6.23	<X>/PLMNList/<X>/PDUInfo/PDUInfoList/<X>/S-NSSAI	12

6.24	<X>/Ext	13

Annex A (informative): Change history	14

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies UE pre-configuration in order to receive the data of broadcast communication service as specified in 3GPP TS 23.247 [3].

The present document defines a management object (MO) that can be used to configure the UE with parameters related to reception of data of broadcast communication service.

The MO is compatible with the OMA Device Management (DM) protocol specifications, version 1.2 and upwards, and is defined using the OMA DM device description framework (DDF) as described in the Enabler Release Definition OMA-ERELD-DM-V1_2 [8].

The MO consists of nodes and leaves conveying UE pre-configuration parameters used for broadcast communication service selection and data reception.

Editor’s note:	The UE pre-configuration parameters and their use by the UE are FFS.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.003: "Numbering, addressing and identification".

[3]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[4]	3GPP TS 26.346: "Multimedia Broadcast/Multicast Service (MBMS); Protocols and Codecs".

[5]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[6]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 1 Standalone".

[7]	IETF RFC 3629: "UTF-8, a transformation format of ISO 10646".

[8]	OMA-ERELD-DM-V1_2: "Enabler Release Definition for OMA Device Management".

[9]	"Unicode 6.3.0, Unicode Standard Annex #15; Unicode Normalization Forms", September 2013. http://www.unicode.org.

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

DNN	Data Network Name

MBS	Multicast/Broadcast Services

S-NSSAI	Single Network Slice Selection Assistance Information

TMGI	Temporary Mobile Group Identity

USD	User Service Description

# 4	General description

A UE can support multicast and broadcast service (MBS) that is a point-to-multipoint service in which data is transmitted from a single source entity to multiple recipients either to all users in a broadcast service area (MBS broadcast communication), or to users in a multicast group (MBS multicast communication) as defined in 3GPP TS 23.247 [3]. The UE may support pre-configuration of information for services using MBS.

If the UE is pre-configured with information related to services using MBS, the UE can discover and receive data for services by using the provisioned configuration.

The UE pre-configuration contains a list of PLMNs in which for each PLMN, the following information is configured:

a)	PLMN ID of the PLMN for which the configuration applies;

b)	RAN information based on NR-ARFCN on which the broadcast communication service is available;

c)	list of TMGI, on which the broadcast communication service is available, along with the associated user service description (USD) information for the MBS broadcast service. If TMGI and USD information for the MBS user service is configured for the PLMN selected for broadcast communication service, the UE uses the information configured therein to acquire the broadcast communication service;

d)	list of TMGI, on which the service announcement for broadcast communication service is available along with the associated USD information for the MBS user service announcement service. If TMGI and USD information for MBS user service announcement is configured for a PLMN selected for broadcast communication service, the UE uses the information configured therein to acquire the service announcement for broadcast communication service; and

e)	PDU session information based on DNN and S-NSSAI pair associated with an MBS session.

# 5	Multicast/Broadcast UE pre-configuration

## 5.1	General

The UE pre-configuration MO is used to configure the UE with parameters related to MBS (see 3GPP TS 23.247 [3]).

The MO identifier is: urn:	oma:mo:ext-3gpp-UE-pre-config-MBS:1.0.

Editor’s note:	The MO identifier needs to be registered.

The OMA DM access control list (ACL) property mechanism (see OMA-ERELD-DM-V1_2 [8]) may be used to grant or deny access rights to OMA DM servers in order to modify nodes and leaf objects of the UE pre-configuration MO.

## 5.2	UE pre-configuration MO structure

The structure of the UE pre-configuration MO is shown in figure 5.2.1 and figure 5.2.1 below.

Figure 5.2.1: UE pre-configuration MO

Figure 5.2.2: PLMNList node

# 6	UE pre-configuration MO parameters

## 6.1	General

This clause describes the nodes and leaves of the UE pre-configuration MO parameters.

## 6.2	Node: <X>

This interior node acts as a placeholder for zero or one accounts for a fixed node.

-	Occurrence: ZeroOrOne

-	Format: node

-	Access Types: Get

-	Values: N/A

## 6.3	<X>/Name

The Name leaf is a name for the UE pre-configuration MO settings.

-	Occurrence: ZeroOrOne

-	Format: chr

-	Access Types: Get

-	Values: <User displayable name>

The User displayable name shall be represented by Unicode characters encoded as UTF-8 as specified in IETF RFC 3629 [7] and formatted using normalization form KC (NFKC) as specified in Unicode Standard Annex #15; Unicode Normalization Forms [9].

## 6.4	<X>/PLMNList

The PlmnList node acts as a placeholder for per-PLMN configuration.

-	Occurrence: ZeroOrOne

-	Format: node

-	Access Types: Get, Replace

-	Values: N/A

## 6.5	<X>/PLMNList/<X>/

This interior node acts as a placeholder for UE pre-configuration parameters for one PLMN.

-	Occurrence: OneOrMore

-	Format: node

-	Access Types: Get, Replace

-	Values: N/A

## 6.6	<X>/PLMNList/<X>/PLMNId

The PlmnId leaf indicates a PLMN identity of the PLMN for which the UE pre-configuration parameters applies.

-	Occurrence: One

-	Format: chr

-	Access Types: Get, Replace

-	Values: <PLMN identity>

The format of the PLMN identity is specified in 3GPP TS 23.003 [2].

## 6.7	<X>/PLMNList/<X>/TMGIConfiguration

The TMGIConfiguration node acts as a placeholder for the TMGI configuration in the PLMN identified by the PlmnId leaf.

-	Occurrence: One

-	Format: chr

-	Access Types: Get, Replace

-	Values: N/A

## 6.8	<X>/PLMNList/<X>/TMGIConfiguration/TMGIListForSA

The TMGIListForSA node acts as a placeholder for the TMGI list and the MBS service announcement information for broadcast communication services.

-	Occurrence: ZeroOrOne

-	Format: node

-	Access Types: Get, Replace

-	Values: N/A

## 6.9	<X>/PLMNList/<X>/TMGIConfiguration/TMGIListForSA/<X>

This node acts as a placeholder for one or more TMGI and the MBS service announcement information for broadcast communication service.

-	Occurrence: OneOrMore

-	Format: node

-	Access Types: Get, Replace

-	Values: N/A

## 6.10	<X>/PLMNList/<X>/TMGIConfiguration/TMGIListForSA/<X>/TMGI

The TMGI leaf indicates a TMGI for the MBS service announcement information for broadcast communication service.

-	Occurrence: One

-	Format: chr

-	Access Types: Get, Replace

-	Values: <TMGI>

The format of the TMGI is specified in 3GPP TS 23.003 [2].

## 6.11	<X>/PLMNList/<X>/TMGIConfiguration/TMGIListForSA/<X>/USD

The USD leaf provides MBS service announcement information for MBS service announcement service for broadcast communication service.

-	Occurrence: ZeroOrOne

-	Format: chr

-	Access Types: Get, Replace

-	Values: <USD>

The format of the USD is defined in 3GPP TS 26.346 [4].

## 6.12	<X>/PLMNList/<X>/TMGIConfiguration/TMGIListForService

The TMGIListForService node acts as a placeholder for the TMGI list and the MBS user service announcement information for the MBS user service for broadcast communication services.

-	Occurrence: ZeroOrOne

-	Format: node

-	Access Types: Get, Replace

-	Values: N/A

## 6.13	<X>/PLMNList/<X>/TMGIConfiguration/TMGIListForService/<X>

This node acts as a placeholder for one or more TMGI and MBS user service announcement information for the MBS user service for broadcast communication services.

-	Occurrence: OneOrMore

-	Format: node

-	Access Types: Get, Replace

-	Values: N/A

## 6.14	<X>/PLMNList/<X>/TMGIConfiguration/TMGIListForService/<X>/TMGI

The TMGI leaf indicates a TMGI for broadcast communication service.

-	Occurrence: One

-	Format: chr

-	Access Types: Get, Replace

-	Values: <TMGI>

The format of the TMGI is specified in 3GPP TS 23.003 [2].

## 6.15	<X>/PLMNList/<X>/TMGIConfiguration/TMGIListForService/<X>/USD

The USD leaf provides an USD for broadcast communication service using MBS. The USD leaf provides the MBS user service announcement information for the MBS user service corresponding to the broadcast communication service.

-	Occurrence: ZeroOrOne

-	Format: chr

-	Access Types: Get, Replace

-	Values: <USD>

The format of the USD is defined in 3GPP TS 26.346 [4].

## 6.16	<X>/PLMNList/<X>/RANInfo

The RANInfo node acts as a placeholder for the RAN-specific information.

-	Occurrence: ZeroOrOne

-	Format: chr

-	Access Types: Get, Replace

-	Values: N/A

## 6.17	<X>/PLMNList/<X>/RANInfo/<X>

This node acts as a placeholder for one or more NR ARFCN values of one or more MBS frequencies.

-	Occurrence: OneOrMore

-	Format: chr

-	Access Types: Get, Replace

-	Values: N/A

## 6.18	<X>/PLMNList/<X>/RANInfo/<X>/NRARFCN

The NRARFCN leaf indicates the NR-ARFCN value of one MBS frequency.

-	Occurrence: One

-	Format: int

-	Access Types: Get, Replace

-	Values: <NRARFCN>

The value of the NRARFCN is a 32-bit long unsigned integer. The format of the NR-ARFCN is specified in 3GPP TS 38.101-1 [5] and 3GPP TS 38.101-2 [6].

## 6.19	<X>/PLMNList/<X>/PDUInfo

The RANInfo node acts as a placeholder for the PDU session-specific information.

-	Occurrence: ZeroOrOne

-	Format: chr

-	Access Types: Get, Replace

-	Values: N/A

## 6.20	<X>/PLMNList/<X>/PDUInfo/PDUInfoList

The PDUInfoIList node acts as a placeholder for pairs of DNN and S-NSSAI values that indicate which PDU session is associated with an MBS session.

-	Occurrence: ZeroOrOne

-	Format: node

-	Access Types: Get, Replace

-	Values: N/A

## 6.21	<X>/PLMNList/<X>/PDUInfo/PDUInfoList/<X>

This node acts as a placeholder for one or more PDU session-specific information values.

-	Occurrence: OneOrMore

-	Format: chr

-	Access Types: Get, Replace

-	Values: N/A

## 6.22	<X>/PLMNList/<X>/PDUInfo/PDUInfoList/<X>/DNN

The DNN leaf indicates the DNN value of one PDU session associated with an MBS session.

-	Occurrence: One

-	Format: chr

-	Access Types: Get, Replace

-	Values: <DNN>

The format of the DNN is defined by 3GPP TS 23.003 [2] in clause 9A.

EXAMPLE:	mycompany.mnc012.mcc340.gprs

## 6.23	<X>/PLMNList/<X>/PDUInfo/PDUInfoList/<X>/S-NSSAI

The S-NSSAI leaf indicates the S-NSSAI value of one PDU session associated with an MBS session.

-	Occurrence: One

-	Format: int

-	Access Types: Get, Replace

-	Values: <S-NSSAI>

The format of the S-NSSAI is defined by 3GPP TS 23.003 [2] in clause 28.4.2

Editor's note:	The need of a TMGI leaf for the DNN+S-NSSAI is for future study.

## 6.24	<X>/Ext

The Ext is an interior node for where the vendor specific information about the UE pre-configuration MO is being placed (vendor meaning application vendor, device vendor etc.). Usually the vendor extension is identified by vendor specific name under the ext node. The tree structure under the vendor identifier is not defined and can therefore include one or more non-standardized sub-trees.

-	Occurrence: ZeroOrOne

-	Format: node

-	Access Types: Get

-	Values: N/A

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-04 | CT1#135-e | C1-223029 |  |  |  | Draft skeleton provided by the rapporteur. | 0.0.0 |
| 2022-04 | CT1#135-e | C1-223030 |  |  |  | Implementing the following p-CR agreed by CT1: C1-223030 | 0.1.0 |
| 2023-03 | CT1#140 | C1-230960 C1-230961 |  |  |  | Implementing the following p-CRs agreed by CT1: C1-230960, C1-230961; and editorials changes from the rapporteur. | 0.2.0 |
| 2023-03 | CT#99 | CP-230183 |  |  |  | TS presented for information and approval | 1.0.0 |
| 2023-03 | CT#99 |  |  |  |  | Version created after approval at TSG CT | 18.0.0 |