Tests vector of Extended AMR-WB decoder

The following document summarizes all the steps needed to validate a bit exact implementation of the Extended AMR-WB (AMR-WB+) decoder.

The test vectors package contains eight (8) bitstreams that must be used to validate the bit exactness of the decoder. All lines of code are covered when decoding these eight bit streams.

These tests vectors are specifically useful for the extended part of the AMR-WB+ decoder (stereo, TCX, frame erasure, switching from ACELP to TCX, switching from AMR-WB+ to AMR-WB, etc.). The AMR-WB part is further covered by another set of tests vectors to validate the bit exactness of AMR-WB part in AMR-WB+.

A script is provided to properly run AMR-WB+ decoder with all these bitstreams. All these bitstreams are also used with a frame erasure file pattern to validate the frame erasure concealment of AMR-WB+.

For evaluating the bit exactness of a tested AMR-WB+ decoder, the instructions below need to be followed:

- Run the batch file once with the reference AMR-WB+ decoder (bin\decoder_fxd)

- Save the synthesis produced in clean_syn and fer_syn

- Modify the executable name at line 31 of testev_dec.bat

- Run the batch file again

- Compare bit exactness between syntheses.

Note: Decoder_fxd (in \bin) is built with a patch described in readme.doc and found in wseg_snr\conformance_code_patch\c-code_tested-amr-wb-e\decoder_fx