Extended AMR-WB (AMR-WB+) objective conformance measures

Note:

The reference fixed-point decoder includes a bug correction in file dec_hf_fx.c. The CR corresponding to this bug correction will be presented at the 3GPP SA4#36 meeting in September 2005. The bug consists of an improper scaling of a threshold, and affects Lines 190 and 197 in file dec_hf_fx.c. The patch is illustrated in wseg_snr\conformance_code_patch\c-code_tested-amr-wb-e\decoder_fx under FUTUR_BUG_CORRECTION compiler flag.

Directory description:

wseg_snr

- conformance_code_patch

- c-code_reference-amr-wb-e : Contains the detailed code modification procedure to follow when integrating conformance measure tools in the reference AMR-WB+ encoder (ex. 26.304)

- c-code_tested-amr-wb-e: Contains the detailed code modification procedure to follow when integrating conformance measure tools in the AMR-WB+ encoder under test (ex. 26.273)

- Audio

Contains example audio files that can be used to validate weighted segmental SNR (wSNR) conformance measures.

- Bin

Contains the fixed-point and floating-point encoder executables compiled with conformance code patch enabled.

- Res

To be use as an output directory by the batch file. This directory will contain all wSNR files produced by the fixed-point and the floating-point encoders. A simple program is provided to verify compliance between floating-point wSNR and fixed-point wSNR.

Peaq

This directory structure contains example scripts allowing to check the conformance of test decoder/encoder implementations according to the objective PEAQ-ODG criterion that are defined in 26.274. Further information is found in readme.txt files in subdirectories.

Contents of subdirectories to peaq:

- audiodir

test items for PEAQ ODG tests

- bin

reference and test executables and tools. This directory contains pre-compiled reference and example test executables. Among them a fixed-point decoder implementation containing the bug fix described in the top of this document.

- encoder_tests

test scripts for encoder conformance testing

- decoder_tests

test scripts for decoder conformance testing

- patches

code patches required for particular encoder tests