| 3GPP TR 26.813 V0.2.1 (2023-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects ; Multimedia Codecs, Systems and Services; Avatar Representation and Communication; (Release 18 ) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	6

4	Overview	6

5	Use Cases	7

6	Avatar Representation and Animation Formats	7

7	Reference Architecture	7

8	Avatar Integration into 3GPP Services and Enablers	7

8.1	Introduction	7

8.2	Signalling of Avatars	7

8.3	Transport of Avatars	7

8.4	QoS Support	7

8.5	Split Rendering of Avatars	7

9	Security and Privacy	7

10	Conclusions	7

Annex A (normative): Example Avatar Representation and Animation Streams	8

Annex <F> (informative): Change history	9

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

This TR documents the representation of Avatars and their integration into 3GPP services and enablers for the immersive communication and the Metaverse.

# 1	Scope

The present document documents formats for representing and animating Avatars. It also provides recommendations on the integration of Avatars in 3GPP services and enablers for immersive communications and the Metaverse.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 22.856: “Feasibility Study on Localized Mobile Metaverse Services”

[3]	ISO/IEC 23090-14:2023, Information Technology – Coded Representation of Immersive Media – Part 14: MPEG Scene Description.

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

Base Avatar

Reference Avatar

Animated Avatar

Avatar Representation Format

Animation Data

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Overview

# 5	Use Cases and Potential Requirements

## 5.1 	UC1: Avatar Communication

TR22.856 Reference Use Case(s)

5.11: IMS-based 3D Avatar Communication 5.26 Use Case on IMS-based 3D Avatar Call Support for Accessibility Use Case

Description:

This use case is about one-to-one and multi-party communication with spatial audio rendering, where avatars and audio of each participant in avatar call are transmitted and spatially rendered in the direction of their geolocation. An avatar call is similar to a video call in that both are visual, interactive, provide live feedback to participants regarding their emotions, attentiveness and other social information. Each participant is equipped with display devices (phones, AR glasses, etc.) with external or built-in headphones. 3D audio can be captured and transmitted instead of mono, which leads to enhancements when sharing the audio experience. A potential user experience for a one-to-one communication is described as a user story: User B is wearing AR glasses while reading a book before she goes to bed. The display on her glasses indicates an incoming call from User A who is using his phone. User B is in her bedroom and it is late at night. She prefers to maintain the privacy of her room but still wants the answer the call. She decides to take the call as an avatar-based call. The call is initiated and a pre-recorded 3D representation (avatar) of her using a different outfit is rendered on User A's phone. She also has a choice of using a full body or a head-only avatar. The sensors on her glasses detect her heads movement and her avatar is animated accordingly. As the conversation continues, User B needs to take off her glasses and use her phone in handset mode. The transition is seamless, and User A is unaware of it as User B's avatar is now animated through the movement sensors on her phone and she continues to appear attentive from his perspective. After establishing an avatar call, the information captured by the sensors and cameras on User B's terminal is processed locally to encode and animate her avatar. Alternatively, depending on the terminal's capabilities, the captured information may be transmitted over an uplink to an AS associated with the established session that performs the encoding. In either case, the encoded representation is sent to User A's terminal for rendering either as a 2D video or a 3D avatar. Note 1: It is advantageous to process the data acquired by the UE's sensors locally on the UE itself to reduce the uplink data requirements and to preserve the confidentiality of the captured data. Note 2: A possible variation of this scenario is a caller interacting with a computer-generated system where an avatar is used to generate an appearance for a simulated entity with whom the user communicates. A potential use experience in a multi-party scenario is described in the following user story: User A is taking the train and is running late for his meeting with User B and User C. User B is working remotely from his home using his HMD, while User A is using the AR glasses and headphones available in the meeting room at the office. User A has his AR glasses with him, and they are tethered to his phone. User A starts a multi-party call with User B and User C, where each of them gets to see a photo-realistic digital representation (avatar) of the other two rendered on his display. While User A and User A see an augmented avatar, User B can see his colleagues' avatars within a virtual meeting room. At some point during the call, User A wants to share the display from his laptop, which is also tethered to his phone, to show the charts for last month's sales. The screen contents are projected in the virtual meeting room that User B sees on his HMD and are also augmented as an overlay on User A's glasses. The virtual meeting room that User B is participating in is setup and managed by an AS that is instantiated by the 5G network. Sensor information from the terminals of the participants is sent to the server to generate their animated avatars and the avatars are then sent to User A and User A's terminals for overlaying and are also included as part of the scene description for the virtual environment that is sent to User B's terminal. Another aspect in this use case is to enable voice driven refined avatar animation. It is well known that when human beings speak, there are associated visible facial expression movements (including lips movement), hand gestures and body movements. Refined facial expressions can be derived from talker’s voice, and the movement of the lips is completely synchronized with the voice content spoken by the talker. The approaches to voice-to-motion generation can differ based on the specific implementation, including rule-based motion map and learning-based motion generation, ASR (automatic speech recognition), etc. Accurately capture such detail information on the transmitting side and render them on the receiving side will certainly enhance avatar communication user experience. Also, human facial expression associated with speech can carry nuances otherwise not easy to convey, such as feeling, anger, command, seriousness. The natural movement of the hands with the speaker's voice can create a coordinated virtual avatar, making communication more friendly and realistic. For avatar communications to get closer to true human behaviour, voice associated facial expression and body movement seem to be necessary. In addition, it is well documented that talker’s lip movements can enhance intelligibility [1]. This is the main reason that normal people can understand the conversation better from a talker at distance while in a noisy environment if they can see each other, not to mention that trained lip-reading experts can understand most conversation by just looking at the talker’s lip movements. It is therefore beneficial to use refined voice driven avatar animation. A potential user experience is described below: Daniel is calling his sister Michelle as usual using avatar based communication method on his smartphone. After some typical conversations between them, Michelle brings up a subject that bothers her and makes her somewhat sad. Luckily Daniel is equipped with voice driven technology enabling his facial expression and body movements to be sent to his avatar that Michelle is looking at. Upon seeing her brother’s exaggerated funny facial expressions and body movements through the rendered avatar and consolidating verbal conversation, Michelle is quickly recovered from her mood and enjoys her experience talking with her brother over the avatar communication method. Another potential use experience in a noisy environment is described in the following user story: User B was watching a real time sport event using his XR HMD with immersive video/audio capability. During the game, his brother Tom called in an avatar session to tell him some important news. Thanks to the accurate voice driven avatar animation with lip movements, User B was able to understand Tom without having to stop watching the sport event in order to understand his brother. Another potential user experience for avatar sharing during a one-to-one communication is described as a user story: User A and User B are having a normal video (or voice) call talking about online cloth shopping. User B found some nice dresses for User A, but is not sure whether the size or style can fit User A well. During the call, User B invokes a side channel in order to share the expected look of the dresses on top of User A’s avatar. To do that, User B first retrieves User A’s lifelike avatar (under proper authorization) then puts on the digital representation of a selected dress (as a digital asset) from the online shop. A new look of the User A’s avatar is generated by the real-time composing function provided by the operator and rendered to the end devices of User A and User B simultaneously. User A may do some modification about the accessories on the avatar as well. User A and User B can make decision easily together with the help of the vivid look of the avatar updated with the selected dresses and accessories. It’s noted that the avatar sharing, modification and composing can be used as assisting information alongside an ordinary one-to-one multimedia communication (voice or video). It is also straightforward to extend the story to cover the multi-party case or the case where participants are already using avatars as their digital representations.

Categorization

Preconditions

Potential Requirements

In addition to relevant requirements from previous use cases, the following requirements shall apply: - An interoperable format for a user’s digital representation (avatar) and for animation data (for the avatar). - Real time transfer of animated user digital representations, animation data, and speech data. - The ability to generate, encode, and decode 3D digital representations (avatars) on the UE to support confidentiality of the data used to produce the 3D avatar. - Synchronization of avatar representation with immersive audio. - The ability to generate animation data from various media (e.g., video, text, etc.) and to animate the avatar using this data (e.g., to express behavior, movement, emotions, etc.) on the UE or in the network. - The ability to select part of an avatar representation (e.g., face only vs. full body vs. head and torso, etc.). - The ability to switch between alternative representations (e.g., from one face to another, from partial to full). - The ability to dynamically update some characteristics of a representation (e.g., from one color of the shirt to another). - The ability to integrate one or more avatars in a scene with proper anchoring, lighting, and spatial awareness (AR) and interaction (XR). - Adaptation Requirements: - Scalability of the avatar representation and applied animation to adapt to bandwidth limitations. - Scalability of the avatar representation and applied animation when used in multiparty scenarios. - Supporting bidirectional transitioning between video and avatar media for the parties in the call (e.g., transitioning to a 3D avatar when the communication performance of one or more parties declines to the extent that video is no longer of sufficient quality or even possible). - Supporting bidirectional transitioning between video and avatar media for the parties in the call (e.g., transitioning to an 3D avatar when the communication performance of one or more parties declines to the extent that video is no longer of sufficient quality or even possible). - Supporting the transcoding of locally generated media (e.g., text or video) of a party in the call to before it is rendered for the receiving party (e.g., to express behavior, movement, emotions, etc.). - The ability to transcode from and to avatar representations to other media types (e.g., between text, speech, etc.) and vice versa to support users with disabilities or to adapt to device capabilities. - Real time facial expression and body movements metadata extraction based on ASR/NLP technics. - The ability to generate and encode/decode 2D/3D avatars animation driven by voice on the UE and network - Supporting sharing digital representations (avatars) as assisting information in the call. - Supporting real-time modification and composing of the digital representations (avatars) in the call. Editor’s Notes: - Aspects related to complexity of avatar generation and how photorealistic the avatar should be, are TBD. - What is the level of confidentiality desired (e.g., is it limited to unauthorized use, or includes blurring areas of the body)? - Transitioning from avatar to video may be desirable for energy efficiency. Some requirements (e.g., authorization, user access management, etc.) may be handled by other 3GPP WGs and need to be aligned.

Feasibility

## 5.2 	UC2: Multi-party shared experiences

TR22.856 Reference Use Case(s)

5.3 collaborative and concurrent engineering in product design using metaverse services

Description:

Avatars can inhabit virtual workspaces or environments, where team members can interact and collaborate on shared projects. Avatars represent team members within the virtual space, allowing for real-time communication, document sharing, and collaborative editing. This facilitates remote collaboration and enables teams to work together as if they were physically co-located. In this use case, distributed virtual environments (DVEs) allow multiple users from different geographical locations to interact over a network. A DVE is a multi-user virtual reality space that actively support communication, collaboration, and coordination, in which participants are represented by graphical embodiments (avatars) that convey their identity, presence, location, and activities to others. The virtual space facilitates communication, shared exploration of 3D visualizations, and collaborative construction of new content. Users can communicate with each other, and they can interact with other users and with the virtual environment. Examples of collaborative working scenarios where avatars may be utilized include, but not limited to: - Co-Creation and Design Collaboration: Avatars enable collaborative co-creation and design in virtual environments. Team members can use avatars to collectively work on projects, such as architectural designs, product prototypes, or 3D models. Avatars allow for real-time interaction with virtual objects, enabling users to manipulate and discuss designs collaboratively. - Training and Simulations: Avatars can be used for collaborative training and simulations. Team members can assume avatar identities within virtual training scenarios, engaging in collaborative learning experiences. Avatars can represent subject matter experts, trainers, or team members in role-playing exercises, enhancing collaboration and knowledge transfer within a simulated environment. - Classroom: A teacher and a group of students are participating in a virtual class room, all are represented with avatars. Each student can not hide the avatar facial and body expression of himself/herself from the teacher, i.e. the teacher can look at one student’s avatar and know if s/he is paying attention to the class or doing something else, or taking a nap for instance. Each student can tailor his/her avatar's facial and body expression for other students, i.e. s/he can hide or simplify their avatars and their animation from the other students, e.g. if a student dozes off, s/he doesn’t want others to notice. - Social gathering of teenage friends: Teenage avatars are similar to avatars in other cases with one difference. Teenagers are sensitive to their appearance and may not like some of their own facial and body language expression habits. When using avatars, a teenager can decide to filter out one or more facial or body language expressions and optionally replace them with another expression. A potential user experience is described as a user story: User A is working in the engineering office. She has just completed here latest design and wants to demonstrate it to User B who is working in the factory. She puts on her HMD and haptic gloves and logs in to her company's office in the metaverse using her credentials. After authentication, she is now in the virtual office space and she rings User B, whose video appears on a virtual display. User B can see User A's avatar on his phone's display. User A then asks him to join her in the virtual office to show him what she has been working on and get his feedback. User B puts on his HMD and gloves and joins her. Both User A and User B can now see each other's avatar in the virtual office, where the avatars are seated facing each other across a table. User A loads her design from the virtual panel in front of her and a 3D model of the design suddenly appears on the table. User B wants to take a closer look at the model, so he leans towards it and picks it up. He is now able to feel the surface of the model and inspect it from different angles and he identifies an issue with one of the parts. They need a third opinion, and they contact Layla from another department, who specializes in this particular function. Layla is only able to join the call her AR glasses and headphones, which are tethered to her phone. Layla joins their meeting, and her avatar appears to User A and User B in the virtual office. While she is unable to see the complete virtual office. The avatars of User A and User B, along with the 3D model of User A's deign, are overlayed on her glasses display.

Categorization

Preconditions

Potential Requirements

The following requirements apply in addition to relevant requirements of Use Case 1: - Generating and maintaining a shared digital virtual environment. - User identity management and data security. - Interoperable representation of avatars - The ability to access and load pre-generated digital representations. - Capturing and sending user pose and motion information uplink to the network. - Visual coding and transmission of avatars. - Transmission and potentially coding of motion (animation) data. - The ability to encode and transport sensory information to and from the UE devices, possibly attached to the user’s body - Synchronizing multiple data flows from multiple UEs associated with one user. - QoS requirements that need to be fulfilled in order for the users’ QoE to be satisfactory. - Delivering secure avatar expressions end-to-end for a certain audience - Capability for filtering certain avatar expressions and/or replacing them at the network for some audience - Support the capability of filtering or replacing a selected subset of face and body expressions when generating the avatar animation.

Feasibility

## 5.3 	UC3: Multi-user Gaming

TR22.856 Reference Use Case(s)

5.6 Use Case on Mobile Metaverse for Immersive Gaming and Live Shows

Description:

Avatars allow players to express their individuality, represent themselves in virtual worlds, and engage in multiplayer interactions. They enhance the visual aspect of gaming and create a more immersive and engaging player experience. A few examples are listed here below: - Player Representation: Avatars allow players to create digital representations of themselves within games. Players can customize their avatars' appearance, clothing, and accessories to reflect their preferences and identities. Avatars serve as the virtual embodiment of players, enabling them to navigate and interact within the game world. - Immersive Gaming Experience: Avatars enhance immersion in gaming by providing a visual representation of the player within the game environment. Players can see their avatars perform actions, movements, and interactions in real-time. Avatars contribute to a sense of presence and agency, making the gaming experience more immersive and engaging. - Personalization and Progression: Avatars can be customized and upgraded as players progress in games. Players can unlock new customization options, abilities, or items for their avatars as they achieve milestones or complete in-game challenges. Avatars evolve alongside players, reflecting their progress and accomplishments. - Competitive E-sports: In e-sports, avatars are used to represent players or teams in competitive matches. Players control their avatars during tournaments or matches, showcasing their skills and strategies. Avatars serve as the visual representation of players' actions, making e-sports broadcasts more engaging and visually appealing. - Virtual Reality (VR) Gaming: In VR gaming, avatars play a crucial role in providing a virtual presence for players. Avatars represent players within the virtual environment, mimicking their movements and actions. VR avatars enhance the feeling of immersion and embodiment, making the VR gaming experience more realistic and interactive. In this use case, a metaverse mobile gaming service is deployed in the cloud or on an edge server for providing an immersive gaming experience where the players can interact with each other over the 5GS. Players are represented by their avatars and are able to communicate with each other whether they are in close proximity or not. Moreover, the players can interact with each other as well as with other objects in the game's virtual world. This interaction may also wearables that the users have on their hands and bodies which provide haptic feedback in response to actions and interactions that the users are part of. In some gaming scenarios, other participants may also be allowed as spectators where their avatars are rendered as members of the audience in the virtual environment of the game. Different rights may be granted to the avatars of the participants based on their roles (e.g., player, audience, etc.). The terminals of the participants are equipped with sensors and cameras and are connected to control devices to capture their movements and interactions. The captured sensing data are sent to the cloud or the edge server which use this information for coding and rendering to generate the digital representations (avatars). Some avatars may not be user controlled but they are software agents controlled by the game engine. A potential user experience is described as a user story: User B logs in to the metaverse from her living room using her HMD and starts an online shooting game. She is using the controllers connected wirelessly to the HMD to play the game and she is wearing a haptics body suit. A session is established between her HMD and the game server in the network, and she can now select the game mode. She selects a multi-player mode and invites User C and User D to join her. User C and User D are using AR glasses tethered to their phones to join the game and are able to see User B's, as well as each other's, avatar augmented to their surroundings. They opt for using hand gestures to play the game. A fourth avatar, User A, controlled by the game engine appears in each of the player's displays. User B teams with User C while User D and User A are assigned as an opposing team. At some point during play, User B senses a vibration in her body suit and she realizes that her avatar was hit in the game.

Categorization

Preconditions

Potential Requirements

In addition to relevant requirements from the previous use cases, the following requirements shall apply: - Sufficient access controls to ensure age-appropriate content. - Support the transmission of uplink sensor data (raw or processed, e.g., pose, animation data, actions, etc. and downlink feedback information with acceptable packet delay and bandwidth for real-time interaction. - The ability to obtain the location and gestures of the players with reasonable 3D positioning accuracy. - Low latency for interaction and motion data transmission and for rendering. - Minimizing the asynchrony between different modalities (audio, visual, and tactile) to maintain good QoE. The synchronization thresholds are described in [Ref. 49 in TR 22.856] and are listed below: - audio-tactile stimuli: 12 ms when the audio comes first and 25 ms when the tactile comes first to be perceived as being synchronous. - visual-tactile stimuli: 30 ms when the video comes first and 20 ms when the tactile comes first to be perceived as being synchronous. - audio-visual stimuli: 20 ms when the audio comes first and 20 ms when the video comes first to be perceived as being synchronous. Note: The obtained results vary, depending on the kind of stimuli, biasing effects of stimulus range, the psychometric methods employed, etc. - KPI requirements related to the potential requirements [1]: - end-to-end latency: [5~20] ms - service bit rate (user experienced data rate): [1~1000] Mbit/s - positioning accuracy: < 1 m

Feasibility

## 5.4 	UC4: Avatar Generation, Storage, and Access

TR22.856 Reference Use Case(s)

5.13 Use Case on Digital Asset Container Information Access and Certification 5.15 Use Case on Access to Avatars 5.16 Use Case on Virtual Store in a Mobile Metaverse Marketplace

Description:

For avatar services, users need a digital representation to represent themselves. Digital representations may be generated by the service provider, and can be created using an app installed on the user's device or by functions provided by the 5G network. When a digital representation is generated on a user's device, it may be pre-uploaded and stored in the network or transmitted to the other party via the network before the start of avatar service. Users may update the digital representation they have stored in the network or transmitted to others. User A wants to engage in an AR avatar service/experience with User B and C in User A's living room. To do this, User A installed an avatar service app a few days ago and followed the app's instructions to capture videos of neutral expressions, smiling expressions, and reading scripts to create User A's digital representation. Upon pre-checking before using the avatar service, User A confirmed that the digital representation could perfectly represent User A's facial expressions and body gestures. The avatar service app recommended pre-uploading the digital representation, taking its size into account, and User A accepted this recommendation. User A may generate several versions of digital representations for different purposes of usage, then upload them with different names, tags, roles or other attributes for future access. User A initiated an avatar service use case (e.g. UC 1, 2, or 3) with User B and C, both of whom also accepted and joined the service, and soon appeared in User A's living room. If multiple avatars were pre-uploaded, User A may choose one of the avatars that fit the target avatar service by searching (or browsing) based on associated attributes of the avatars from the repository. During the avatar service, User A noticed some awkwardness in one of User A's hands and instructed the app to update that part. The app captured the hand again, and the network updated the digital representation in real-time, providing the updated digital representation to User A, B, and C simultaneously. In some scenarios, such as business activities, authorization of avatar usage rights is needed for commercial or other purposes. Therefore, the 5G system needs to support management and authorization of avatar usage rights. The owner of the avatar is expected to be responsible for the speech and behavior of his/her avatar. An individual or an enterprise has to be authorized by the owner of an avatar before using the avatar especially in business activities. Company A registers with the MNO B as an enterprise customer, while User C registers as an individual customer of MNO B. MNO B assigns IDs for Company A and User C respectively. User C registers her personal avatar with MNO B. The avatar is lifelike and mapped to User C’s ID in real world. So MNO B identifies and stores the avatar and its ID, also associates with the avatar’s owner, User C’s ID. Company A sends a request for the usage rights of User C’s avatar that is managed by MNO B. MNO B sends a request to User C’s to ask for authorization of the avatar to be used in the mobile metaverse services. After being confirmed by User C, Company A’s usage rights of avatar has been authorized. MNO B updates the system with the information that Company A has the usage rights of User C’s avatar.

Categorization

Preconditions

Potential Requirements

In addition to relevant requirements from previous use cases, the following requirements shall apply: From SA1: [PR 5.24.6-1] Subject to regulatory requirements, user’s consent and operator’s policy, the 5G system shall support mechanisms to identify an avatar and associate the avatar with a subscriber (i.e. the owner of the avatar). [PR 5.24.6-2] Subject to regulatory requirements, user’s consent and operator’s policy, the 5G system shall be able to authorize the avatar to be used in mobile metaverse services. [PR 5.24.6-3] Subject to regulatory requirements, user’s consent and operator’s policy, the 5G system shall provide time-bound authorization services for an avatar to be used in mobile metaverse services. [PR 5.24.6-4] Subject to regulatory requirements, user consent and operator’s policy, the 5G system shall be able to support mechanisms to manage the authorization information about the use of an avatar in mobile metaverse services (e.g. the applied time-bound authorization services, the authorized users). [PR 5.24.6-5] Subject to regulatory requirements, user consent and operator’s policy, the 5G system shall be able to identify the subscriber who has the right to use an avatar in mobile metaverse services. Re-written - Support the generation, update, and animation (by the UE and/or network media functions) of a user’s digital representation(s) captured from the user devices (e.g., RGB or RGB-D video/images, LiDAR, etc.), subject to user consent, operator policy, and regulatory requirements - Support for avatar metadata that allows for parental control and user access and management. - Support discovery and negotiation to determine the availability of the network functions supporting the media format(s) and media processes related to avatar processing (e.g., generation, storage, update, animation, and AI-based animation generation, etc.). - Support the upload, storage, search, transmission, and update of a user's digital representation while ensuring the consistency of the components of the digital representation, subject to user consent, operator policy, and regulatory requirements. - Support avatar identification and mapping to a subscriber. - Support management and authorization of avatar usage right. - The ability to identify the subscriber who has the right to use an avatar.

Feasibility

## 5.5	UC5: Artificial Intelligence-Based Avatar

TR22.856 Reference Use Case(s)

5.12 Use Cases on Virtual humans in metaverse 5.17 Use Cases on Work delegation to autonomous virtual alter ego

Description:

An AI avatar is a digital character powered by artificial intelligence, which lives in a virtual setting, like a game, social network or online world. More frequently avatars are designed as human-like bots that can be controlled by real human using AI technologies and can easily engage with real humans and maintain relationship - to varying degrees - with them. This use case proposes a communication with an autonomous virtual alter ego, which is an AI-based digital representation acting autonomously on behalf of a user herself/himself in the mobile metaverse services. For example, user's autonomous virtual alter ego autonomously sends a mail to clients on user’s behalf. Also, the alter ego can autonomously communicate with the user, other physical users, and other alter ego by using the network capabilities based on the user’s 3GPP subscription. Therefore, the use of network by the alter ego has to be captured correctly by the network from charging point of view. NOTE: The term "autonomous virtual alter ego" means an AI-based digital representation behaving autonomously on behalf of a user herself/himself in the mobile metaverse services. All the experience and knowledge performed both in physical world and metaverse will be shared between the alter ego and its user, thus creating more than double the opportunities to play multiple roles simultaneously. This autonomous virtual alter ego concept aims to improve emotional well–being, health, and life satisfaction by enabling users to perceive many opportunities in life, such as balancing work and family and participating in many communities simultaneously. A potential user experience is described as a user story: User A faces a scheduling conflict between an important F2F client meeting and an internal web meeting. To tackle this problem, he connects to the alter ego application via his UE and asks his virtual alter ego (e.g., AI avatar) to participate in the internal web meeting on his behalf. The alter ego first checks and evaluates whether the network resources on 5GS and computing resources on the alter ego server are sufficient to execute the requested tasks. If deemed feasible, the enabler server proceeds to train the alter ego and execute the agree-upon tasks. Otherwise, it proposes alternative task examples to User A, based on its current resources. The alter ego accesses the web meeting server and the necessary meeting information via internet or 5GS by receiving the permission from User A and web meeting server. During the meeting, the alter ego interacts with other attendees, including humans and other virtual alter egos, by explaining, asking questions, or making comments. User A may choose to monitor the meeting participated by his alter ego, and instruct (in the form of voice, text, gesture, etc.) the alter ego to perform certain actions on important issues (e.g. raising hands for voting or commenting). After the meeting, the alter ego autonomously reports back to User A, allowing User A to provide feedback and new instructions if necessary. Additionally, the charging information is collected and linked to user’s charging information to charge the user subscribing the alter ego service.

Categorization

Preconditions

Potential Requirements

In addition to relevant requirements from previous use cases, the following requirements shall apply: - AI/ML model transfer in frameworks documented in TR 23.700-80 [7]. - Securely registering, storing and updating the AI-based digital representation (e.g., AI avatar) for a user. - Assisting the authentication of a third party to use subscriber’s digital representation (e.g., AI avatar) and to access multimedia communication services on behalf of the subscriber. - Providing storage space that a user can use to store application specific data and information about himself in the network. - Supporting transmitting multi-modal instructions to the AI-based digital representation (e.g., AI avatar) for real-time interaction. - Collecting charging information associated with communication involving a digital representation associated with the subscriber.

Feasibility

## 5.6 	List of Potential Requirements

# 6	Avatar Representation and Animation Formats

## 6.1 	General

## 6.2 	Existing Avatar Representation Formats

### MPEG Reference Avatar

#### Body Model

##### 6.2.1.1.1 	Overview

The MPEG-I Scene Description reference body avatar (Figure 1) comes with its own mesh topology modeled as a female (right side) or as a male body (left side). The body base mesh is composed of three levels of detail (detailed in Section 6.2.1.1.2). As shown in Figure 1, the density of vertex is not the same for the body and the face, because the later requires more accuracy for realism.

Figure 1 Full body mesh topology (right : female body, left : male body). The gender-neutral face is zoomed for better visualization.

##### 6.2.1.1.2 	Levels of Detail

The MPEG-SD reference body avatar is available in three levels of detail (mesh resolutions): low, medium and high (see Figure 2). This ensures that the mesh topology is supported on different runtime platforms (e.g., mobile phone, PC, HMD, etc.). The characteristics of each of these levels are as follows:

- High Resolution (referred to as Morgan_HD)

  - 53,695 vertices and 56,496 quad faces (106,952 triangular faces).

- Medium Resolution (referred to as Morgan_MD).

  - 13,225 vertices and 13,196 quad faces (26,356 triangular faces)

  - Corresponds to 25% of the HD mesh.

- Low Resolution (referred to as Morgan_SD)

  - 5,366 vertices and 5,300 quad faces (10,571 triangular faces).

  - Corresponds to 90% of the HD mesh,

The mesh decimation ensures that vertices positions are the same from a lower level to a higher level. Meaning that all vertices of the SD mesh are all on the MD mesh and all vertices of the MD mesh are on the HD mesh.

Figure 2 Levels of detail of Morgan - From left to right: High (50k), Medium (12k) and Small (5k).

##### 6.2.1.1.3 	Model Body Semantics

The body is divided into 47 semantical body regions areas allowing to directly refer to them for specific use cases and applications. Figures 3, 4, and 5 describe these 47 areas for the front, back, and head body parts. The corresponding semantics and vertex coordinates are given by Table 1 in [3], which provides the correspondence between the semantical regions and the geometric properties of the body model. The vertices and faces are in-order to facilitate interpretation. The example provided corresponds to the "High Resolution" level (Morgan_HD). This allows easy access to parts of the body by any applications, even without knowing the model.

Figure 3 Front view of body semantic areas.

Figure 4 Back view of body semantic areas.

Figure 5 Front-view of the head. Top line: segmentation areas, bottom line: wireframe areas. (Left) complete head view with the face (white), left eye (red) and right eye (olive green) - (Middle) internal view without the face with the mouth bag (blue) - (Right) view without the face and the mouth bag - upper jaw (light green) and lower jaw (purple).

##### 6.2.1.1.4 	Base UV Maps

The Morgan model contains multiple UV maps coordinates for all geometric body parts and all LODs. This is based on U-Dimension Mapping (UDIM), which is an enhancement to the UV mapping and texturing workflow that allows to split the UV map into several tiles, each with a resolution adapted to the corresponding region of the mesh geometry. The concept is illustrated in Figure 7 in [3] accompanied by the labelling table below the figure. Each UV tile represents a different texture image with its own UV coordinates.

##### 6.2.1.1.5 	Model Skeleton

For the purpose of re-animation and manipulation of Morgan, it is necessary to include skeletal and rig information. Therefore, skeleton joints are also provided in their usual form, i.e., the joints are identified by a naming convention and follow a hierarchical representation to ease manipulation and animation generation techniques. Figure 6 illustrates the complete skeleton hierarchy and the naming convention adopted for Morgan. For simplification and general compatibility with other conventions, Table 1 provides a simplified version of the full skeleton model. The full hierarchical skeletal joint description can be found in Table 2 in [3].

Figure 6 Position of the 63 joints of Morgan

Table 1 Morgan’s Simplified and Simplistic skeleton joints nomenclature.

| Simplified Body Hierarchy (24 joints) | Simplistic Body Hierarchy (15 joints) |
| --- | --- |
| Hips — Spine — — UpperChest — — — Shoulder_Left — — — — LowerArm_Left — — — — — Hand_Left — — — — — — ProximalThumb_Left — — — Shoulder_Right — — — — LowerArm_Right — — — — — Hand_Right — — — — — — ProximalThumb_Right — — — — Neck — — — — — Head — — — — — — Eye_Left — — — — — — Eye_Right — — — — — — Jaw — UpperLeg_Left — — LowerLeg_Left — — — Foot_Left — — — — Toes_Left — UpperLeg_Right — — LowerLeg_Right — — — Foot_Right — — — — Toes_Right | Hips — UpperChest — — Shoulder_Left — — — LowerArm_Left — — — — Hand_Left — — Shoulder_Right — — — LowerArm_Right — — — — Hand_Right — — — Head — UpperLeg_Left — — LowerLeg_Left — — — Foot_Left — UpperLeg_Right — — LowerLeg_Right — — — Foot_Right |

##### 6.2.1.1.6 	Skinning

Morgan’s skeleton and full body mesh are linked with skinning weights enabling to animate it using skeleton animation. These skinning weights have been obtained using a two-steps process: automatic linear binding algorithm based on closest distance followed by manual painting corrections by a professional artist. As animation requires highly specific motion types, these weights remain fully modifiable.

#### Face Model

##### 6.2.1.2.1 	Overview

Morgan’s face model consists of a base topology and a gender-neutral head morphology. Figure 7 illustrates the geometric composition of the head and face model. The neutral morphology was obtained from a combination of high-resolution face scans dataset containing male and female subjects registered to a reference mesh topology. In the topology depicted in Figure 7, the head mesh is composed of 36, 584 vertices and 36, 362 quad faces, and 72, 784 triangular faces.

Figure 7 Morgan’s face (right to left: front, back, and right profile).

Figure 8 illustrates the four internal parts of the head model, as follows:

- a mouth bag (Figure 8a)

- an upper (Figure 8b) and lower jaw (Figure 8c)

- two eyeballs (Figure 8d)

Figure 8 Morgan’s face details ((a) mouth bag, (b) upper jaw, (c) lower jaw, (d) eyeballs).

Topological details of these specific parts can be found in Table 1 in [3].

##### 6.2.1.2.2 	Face Blend Shapes

The Morgan model provides a collection of facial blend shapes, as listed in Table 4 in [3]. The facial expression shape naming convention is similar but not identical to the FACS naming convention defined and used in anatomy to classify human facial motions [Ekman et al. 1980]. The shapes are separated into Left (“_L”) and Right (“_R”) components. And sometimes these last components have been split into (“_1”) and (“_2”) sub-components to increase the precision.

##### 6.2.1.2.3 	Facial Landmarks

To facilitate the registration of Morgan’s face against other facial meshes, or the addition of headwear or glasses to the avatar model, a set of 68 semantic facial landmarks is also provided, as illustrated in Figure 9. These landmarks are also useful for retargeting techniques, or to anchor Morgan’s face to a facial performance tracker solution.

Figure 9 xx

##### 6.2.1.2.4 	Teeth Model

The facial rig of Morgan includes an upper and a lower jaw, as illustrated in Figure 12 in [3]. The lower jaw is composed of 10, 440 vertices and 10, 380 quad faces (20, 760 triangular faces). The upper jaw is composed of 11, 107 vertices, 11, 040 quad faces and 22, 080 triangular faces. They both contain separate UV coordinates and associated textures.

##### 6.2.1.2.5 	Eye Model

Morgan comes with a complete eye model depicted in Figure 10. The eye model follows an anatomical eye model and is composed of three elements: the sclera, the iris, and the lens. The polygon count is as follows:

• lens: 201 vertices and 200 quad faces (380 triangular faces)

• iris: 928 vertices and 896 quad faces (1792 triangular faces)

• sclera: 290 vertices and 288 quad faces (576 triangular faces)

Figure 10 Morgan’s eyes model.

The eye model of Morgan comes with two blend shapes for each eye, as follows:

- Pupil_Dilatation: controls the aperture of the pupil of the eye.

- Sclera_Flatness: controls the degree of flatness of the cornea.

# 7	Reference Architecture

The following figure depicts the reference Architecture for Avatar:

Figure 11. Avatar Reference Architecture

The identified Avatar functions are:

- Avatar Storage: an entity that offers storage of base Avatars. This entity may be offered by the 5G System, a 3rd party entity, or the local storage of the user’s devices. The Avatar Storage ensures proper access to the base Avatar and any related data.

- Avatar Animation: depending on the Avatar representation format, this entity retrieves the base Avatar, receives representation format-specific animation data streams, and performs the Avatar animation to produce the animated Avatar that will be used in the rendering process. 
[Note that some animation approaches may not need to rely on the 3D base avatar, instead they directly produce rendered 2D view of the Avatar.]

- Scene Management: creates and composes the shared 3D scene for all participants. It integrates a description of the user’s Avatar and updates its position and orientation based on the user’s pose. The updated scene is shared with all participants.

- Animation data generation: generating animation data from raw signals. The raw signals may come from cameras, microphones, and specialized motion capturing devices, etc. For example, through the current functional element, the video captured by the camera can be converted into facial feature points, and the audio captured by the microphone can be converted into text, etc.

- Base Avatar Generation: generates the Base Avatar from the inputs such as captured video from camera and other sensors information. Note that this might be done online or offline.

# 8	Avatar Integration into 3GPP Services and Enablers

## 8.1	Introduction

## 8.2	Signalling

## 8.3	Transport

## 8.4	QoS Support

## 8.5	Split Rendering of Avatars

## 8.6	Mapping to IMS-based Services

## 8.7	Mapping to non-IMS-based Services

# 9	Security and Privacy

# 10	Conclusions

Annex A (normative):
Example Avatar Representation and Animation Streams

Annex <F> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 25/8/2023 | 125 |  |  |  |  | Initial draft of the TR | 0.1.0 |
| 24/10/2023 | post 125 | S4aV230059 |  |  |  | Added Use Case 5 | 0.1.5 |
| 24/10/2023 | post 125 | S4aV230060 |  |  |  | Updated Use Case 4 | 0.1.5 |
| 17/11/2023 | 126 | S4-231877 |  |  |  | MPEG reference avatar | 0.2.0 |
| 17/11/2023 | 126 | S4-232033 |  |  |  | Avatar reference model | 0.2.0 |
| 17/11/2023 | 126 | S4-231838 |  |  |  | Updates after adhoc calls | 0.2.0 |
| 17/11/2023 | 126 | S4-232024 |  |  |  | Updates to use cases | 0.2.0 |
| 17/11/2023 | 126 | S4-232034 |  |  |  | Updates to use cases | 0.2.0 |
| 17/11/2023 | 126 | S4-231650 |  |  |  | Updates to use cases | 0.2.0 |