| 3GPP TR 26.813 V1.0.1 (2025-02) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects ; Multimedia Codecs, Systems and Services; Avatar Representation and Communication; (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	7

4	Overview	8

5	Use Cases and Potential Requirements	8

5.1 	UC1: Avatar Communication	8

5.2	UC2: Multi-party shared experiences	10

5.3	UC3: Multi-user Gaming	12

5.4	UC4: Avatar Generation, Storage, and Access	14

5.5	UC5: Artificial Intelligence-Based Avatar	15

5.6	List of Potential Requirements	17

6	Avatar Representation and Animation Formats	17

6.1	General	17

6.2	Common Terminology	17

6.3	Existing Avatar Representation Formats	17

6.3.1	MPEG Reference Avatar	17

6.3.2	Skinning and Animation in glTF 2.0	26

6.3.3	Avatar Support in MPEG-I Scene Description	26

6.3.4	MPEG Avatar Representation Format	27

6.4	Animation Approaches and Formats	29

6.4.1	Morph target animation	29

6.5	Existing 2D Avatar Representations	30

6.5.1	Introduction	30

6.5.2	2D Avatar Use Cases	30

6.5.3	2D Base Avatar generation	31

6.5.4	2D Avatar animation	33

7	Reference Architecture	35

8	Avatar Integration into 3GPP Services and Enablers	37

8.1	Introduction	37

8.2	Signalling	37

8.3	Transport	37

8.4	QoS Support	37

8.5	Split Rendering of Avatars	37

8.6	Mapping to IMS-based Services	37

8.7	Mapping to non-IMS-based Services	37

8.7.1	Architecture Mapping	37

8.7.2	Call flows	38

9	Security and Privacy	40

9.1	General	40

9.2	Relevant Protection Techniques	41

9.2.1	3D Watermarking for Mesh-based Avatar Protection	41

10	Conclusions	42

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

This TR documents the representation of Avatars and their integration into 3GPP services and enablers for the immersive communication and the Metaverse.

# 1	Scope

The present document documents formats for representing and animating Avatars. It also provides recommendations on the integration of Avatars in 3GPP services and enablers for immersive communications and the Metaverse.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 22.856: “Feasibility Study on Localized Mobile Metaverse Services”

[3]	ISO/IEC 23090-14:2023, Information Technology – Coded Representation of Immersive Media – Part 14: MPEG Scene Description.

[4]	AFLW (Annotated Facial Landmarks in the Wild), accessible here:  https://paperswithcode.com/dataset/aflw, January 2024.

[5]	The LFPW (Labeled Face Parts in the Wild)，accessible here: https://paperswithcode.com/dataset/lfpw, July 2023

[6]	WFLW (Wider Facial Landmark in the Wild), accessible here: https://paperswithcode.com/dataset/wflw，November 2023

[7]	WG03N1316, “Procedures and Test Formats for Avatar Representation Formats as part of MPEG-I”, MPEG#147, Sapporo, Japan, July 2024.

[8]	Wang, Feng, Hang Zhou, Han Fang, Xiaojuan Dong, Weiming Zhang, Xi Yang and Nenghai Yu. “Deep 3D mesh watermarking with self-adaptive robustness.” Cybersecurity 5 (2021): n. pag.

[9]	Zhu, Xingyu, et al. "Rethinking Mesh Watermark: Towards Highly Robust and Adaptable Deep 3D Mesh Watermarking." Proceedings of the AAAI Conference on Artificial Intelligence. Vol. 38. No. 7. 2024.

[10]	Narendra, Modigari, et al. "Levenberg–Marquardt deep neural watermarking for 3D mesh using nearest centroid salient point learning." Scientific Reports 14.1 (2024): 6942.

[11]	 ITU-T F.748.14 (03/2022), "Requirements and evaluation methods of non-interactive 2D real-person digital human application system".

[12] 	ITU-T F.748.15 (03/2022), "Framework and metrics for digital human application system".

[13] 	Zhou Y, Han X, Shechtman E, et al. Makelttalk: speaker-aware talking-head animation[J]. ACM Transactions On Graphics (TOG), 2020, 39(6): 1-15.

[14] 	Zhao J, Zhang H. Thin-plate spline motion model for image animation[C]//Proceedings of the IEEE/CVF Conference on Computer Vision and Pattern Recognition. 2022: 3657-3666.

[15] 	Liu, Jinglin, et al. "Parallel and High-Fidelity Text-to-Lip Generation." Proceedings of the AAAI Conference on Artificial Intelligence. Vol. 36. No. 2. 2022.

[16]	PyTorch, https://pytorch.org/

[17]	ONNX, https://onnx.ai/

[18]	TensorRT, https://developer.nvidia.com/tensorrt

[19] 	mindspore, https://www.mindspore.cn

[20]	Yilong Liu, et al., “Video-Audio Driven Real-Time Facial Animation”, https://www.cs.sjtu.edu.cn/~shengbin/course/SE/Video-Audio%20Driven%20Real-Time%20Facial%20Animation.pdf

[21]	Diana T. Mosa, et al, “A real-time Arabic avatar for deaf–mute community using attention mechanism”, https://link.springer.com/article/10.1007/s00521-023-08858-6

[22]	3GPP TR 26.927 v0.7.0: “Study on Artificial Intelligence and Machine Learning in 5G”.

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Base Avatar: A personalized avatar representation. The base avatar model conforms to a predetermined avatar representation format.

Reference Avatar: An instance of the avatar representation format that is used as a template that is used to generate the base avatar.

Animated Avatar: The resulting avatar model after applying animation data to the base avatar.

Avatar Representation Format: A format for a representation that describes a model for an avatar.

3D Avatar Representation Format: An Avatar Representation Format that includes the topology of the 3D mesh representing the avatar, the topology of the skeleton used to animate the mesh, as well as additional metadata describing semantic properties of the avatar.

Animation Data: Data used to generate the body pose and facial expressions of the base avatar model at each time frame. Avatar animation data are tied to a given base avatar model.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

ARF	Avatar Representation Format

# 4	Overview

This TR provides a comprehensive analysis of avatar representation and communication in the context of 3GPP services and enablers for immersive experiences. It documents existing avatar representation formats, animation techniques, and integration approaches within IMS and non-IMS based services. The TR lists key use cases, which were derived from the SA1 study, and outlines the associated requirements for real-time rendering, encoding, and transmission of avatar data. It then analyses and documents the different enablers that would be required for the standardization of avatar communication. Additionally, the TR addresses security considerations for the protection against malicious usage of Avatars. Finally, the TR provides conclusions and lays the ground for future normative work to introduce avatar support in 3GPP services and enablers.

# 5	Use Cases and Potential Requirements

## 5.1 	UC1: Avatar Communication

TR22.856 Reference Use Case(s)

5.11: IMS-based 3D Avatar Communication 5.26 Use Case on IMS-based 3D Avatar Call Support for Accessibility Use Case

Description:

This use case is about one-to-one and multi-party communication with spatial audio rendering, where avatars and audio of each participant in avatar call are transmitted and spatially rendered in the direction of their geolocation. An avatar call is similar to a video call in that both are visual, interactive, provide live feedback to participants regarding their emotions, attentiveness and other social information. Each participant is equipped with display devices (phones, AR glasses, etc.) with external or built-in headphones. 3D audio can be captured and transmitted instead of mono, which leads to enhancements when sharing the audio experience. A potential user experience for a one-to-one communication is described as a user story: User B is wearing AR glasses while reading a book before she goes to bed. The display on her glasses indicates an incoming call from User A who is using his phone. User B is in her bedroom and it is late at night. She prefers to maintain the privacy of her room but still wants the answer the call. She decides to take the call as an avatar-based call. The call is initiated and a pre-recorded 3D representation (avatar) of her using a different outfit is rendered on User A's phone. She also has a choice of using a full body or a head-only avatar. The sensors on her glasses detect her heads movement and her avatar is animated accordingly. As the conversation continues, User B needs to take off her glasses and use her phone in handset mode. The transition is seamless, and User A is unaware of it as User B's avatar is now animated through the movement sensors on her phone and she continues to appear attentive from his perspective. After establishing an avatar call, the information captured by the sensors and cameras on User B's terminal is processed locally to encode and animate her avatar. Alternatively, depending on the terminal's capabilities, the captured information may be transmitted over an uplink to an AS associated with the established session that performs the encoding. In either case, the encoded representation is sent to User A's terminal for rendering either as a 2D video or a 3D avatar. Note 1: It is advantageous to process the data acquired by the UE's sensors locally on the UE itself to reduce the uplink data requirements and to preserve the confidentiality of the captured data. Note 2: A possible variation of this scenario is a caller interacting with a computer-generated system where an avatar is used to generate an appearance for a simulated entity with whom the user communicates. A potential use experience in a multi-party scenario is described in the following user story: User A is taking the train and is running late for his meeting with User B and User C. User B is working remotely from his home using his HMD, while User A is using the AR glasses and headphones available in the meeting room at the office. User A has his AR glasses with him, and they are tethered to his phone. User A starts a multi-party call with User B and User C, where each of them gets to see a photo-realistic digital representation (avatar) of the other two rendered on his display. While User A and User A see an augmented avatar, User B can see his colleagues' avatars within a virtual meeting room. At some point during the call, User A wants to share the display from his laptop, which is also tethered to his phone, to show the charts for last month's sales. The screen contents are projected in the virtual meeting room that User B sees on his HMD and are also augmented as an overlay on User A's glasses. The virtual meeting room that User B is participating in is setup and managed by an AS that is instantiated by the 5G network. Sensor information from the terminals of the participants is sent to the server to generate their animated avatars and the avatars are then sent to User A and User A's terminals for overlaying and are also included as part of the scene description for the virtual environment that is sent to User B's terminal. Another aspect in this use case is to enable voice driven refined avatar animation. It is well known that when human beings speak, there are associated visible facial expression movements (including lips movement), hand gestures and body movements. Refined facial expressions can be derived from talker’s voice, and the movement of the lips is completely synchronized with the voice content spoken by the talker. The approaches to voice-to-motion generation can differ based on the specific implementation, including rule-based motion map and learning-based motion generation, ASR (automatic speech recognition), etc. Accurately capture such detail information on the transmitting side and render them on the receiving side will certainly enhance avatar communication user experience. Also, human facial expression associated with speech can carry nuances otherwise not easy to convey, such as feeling, anger, command, seriousness. The natural movement of the hands with the speaker's voice can create a coordinated virtual avatar, making communication more friendly and realistic. For avatar communications to get closer to true human behaviour, voice associated facial expression and body movement seem to be necessary. In addition, it is well documented that talker’s lip movements can enhance intelligibility [1]. This is the main reason that normal people can understand the conversation better from a talker at distance while in a noisy environment if they can see each other, not to mention that trained lip-reading experts can understand most conversation by just looking at the talker’s lip movements. It is therefore beneficial to use refined voice driven avatar animation. A potential user experience is described below: Daniel is calling his sister Michelle as usual using avatar based communication method on his smartphone. After some typical conversations between them, Michelle brings up a subject that bothers her and makes her somewhat sad. Luckily Daniel is equipped with voice driven technology enabling his facial expression and body movements to be sent to his avatar that Michelle is looking at. Upon seeing her brother’s exaggerated funny facial expressions and body movements through the rendered avatar and consolidating verbal conversation, Michelle is quickly recovered from her mood and enjoys her experience talking with her brother over the avatar communication method. Another potential use experience in a noisy environment is described in the following user story: User B was watching a real time sport event using his XR HMD with immersive video/audio capability. During the game, his brother Tom called in an avatar session to tell him some important news. Thanks to the accurate voice driven avatar animation with lip movements, User B was able to understand Tom without having to stop watching the sport event in order to understand his brother. Another potential user experience for avatar sharing during a one-to-one communication is described as a user story: User A and User B are having a normal video (or voice) call talking about online cloth shopping. User B found some nice dresses for User A, but is not sure whether the size or style can fit User A well. During the call, User B invokes a side channel in order to share the expected look of the dresses on top of User A’s avatar. To do that, User B first retrieves User A’s lifelike avatar (under proper authorization) then puts on the digital representation of a selected dress (as a digital asset) from the online shop. A new look of the User A’s avatar is generated by the real-time composing function provided by the operator and rendered to the end devices of User A and User B simultaneously. User A may do some modification about the accessories on the avatar as well. User A and User B can make decision easily together with the help of the vivid look of the avatar updated with the selected dresses and accessories. It’s noted that the avatar sharing, modification and composing can be used as assisting information alongside an ordinary one-to-one multimedia communication (voice or video). It is also straightforward to extend the story to cover the multi-party case or the case where participants are already using avatars as their digital representations.

Categorization

Preconditions

Potential Requirements

In addition to relevant requirements from previous use cases, the following requirements shall apply: - An interoperable format for a user’s digital representation (avatar) and for animation data (for the avatar). - Real time transfer of animated user digital representations, animation data, and speech data. - The ability to generate, encode, and decode 3D digital representations (avatars) on the UE to support confidentiality of the data used to produce the 3D avatar. - Synchronization of avatar representation with immersive audio. - The ability to generate animation data from various media (e.g., video, text, etc.) and to animate the avatar using this data (e.g., to express behavior, movement, emotions, etc.) on the UE or in the network. - The ability to select part of an avatar representation (e.g., face only vs. full body vs. head and torso, etc.). - The ability to switch between alternative representations (e.g., from one face to another, from partial to full). - The ability to dynamically update some characteristics of a representation (e.g., from one color of the shirt to another). - The ability to integrate one or more avatars in a scene with proper anchoring, lighting, and spatial awareness (AR) and interaction (XR). - Adaptation Requirements: - Scalability of the avatar representation and applied animation to adapt to bandwidth limitations. - Scalability of the avatar representation and applied animation when used in multiparty scenarios. - Supporting bidirectional transitioning between video and avatar media for the parties in the call (e.g., transitioning to a 3D avatar when the communication performance of one or more parties declines to the extent that video is no longer of sufficient quality or even possible). - Supporting bidirectional transitioning between video and avatar media for the parties in the call (e.g., transitioning to an 3D avatar when the communication performance of one or more parties declines to the extent that video is no longer of sufficient quality or even possible). - Supporting the transcoding of locally generated media (e.g., text or video) of a party in the call to before it is rendered for the receiving party (e.g., to express behavior, movement, emotions, etc.). - The ability to transcode from and to avatar representations to other media types (e.g., between text, speech, etc.) and vice versa to support users with disabilities or to adapt to device capabilities. - Real time facial expression and body movements metadata extraction based on ASR/NLP technics. - The ability to generate and encode/decode 2D/3D avatars animation driven by voice on the UE and network - Supporting sharing digital representations (avatars) as assisting information in the call. - Supporting real-time modification and composing of the digital representations (avatars) in the call. Notes: - Aspects related to complexity of avatar generation and how photorealistic the avatar should be, are TBD. - What is the level of confidentiality desired (e.g., is it limited to unauthorized use, or includes blurring areas of the body)? - Transitioning from avatar to video may be desirable for energy efficiency. Some requirements (e.g., authorization, user access management, etc.) may be handled by other 3GPP WGs and need to be aligned.

Feasibility

## 5.2	UC2: Multi-party shared experiences

TR22.856 Reference Use Case(s)

5.3 collaborative and concurrent engineering in product design using metaverse services

Description:

Avatars can inhabit virtual workspaces or environments, where team members can interact and collaborate on shared projects. Avatars represent team members within the virtual space, allowing for real-time communication, document sharing, and collaborative editing. This facilitates remote collaboration and enables teams to work together as if they were physically co-located. In this use case, distributed virtual environments (DVEs) allow multiple users from different geographical locations to interact over a network. A DVE is a multi-user virtual reality space that actively support communication, collaboration, and coordination, in which participants are represented by graphical embodiments (avatars) that convey their identity, presence, location, and activities to others. The virtual space facilitates communication, shared exploration of 3D visualizations, and collaborative construction of new content. Users can communicate with each other, and they can interact with other users and with the virtual environment. Examples of collaborative working scenarios where avatars may be utilized include, but not limited to: - Co-Creation and Design Collaboration: Avatars enable collaborative co-creation and design in virtual environments. Team members can use avatars to collectively work on projects, such as architectural designs, product prototypes, or 3D models. Avatars allow for real-time interaction with virtual objects, enabling users to manipulate and discuss designs collaboratively. - Training and Simulations: Avatars can be used for collaborative training and simulations. Team members can assume avatar identities within virtual training scenarios, engaging in collaborative learning experiences. Avatars can represent subject matter experts, trainers, or team members in role-playing exercises, enhancing collaboration and knowledge transfer within a simulated environment. - Classroom: A teacher and a group of students are participating in a virtual class room, all are represented with avatars. Each student can not hide the avatar facial and body expression of himself/herself from the teacher, i.e. the teacher can look at one student’s avatar and know if s/he is paying attention to the class or doing something else, or taking a nap for instance. Each student can tailor his/her avatar's facial and body expression for other students, i.e. s/he can hide or simplify their avatars and their animation from the other students, e.g. if a student dozes off, s/he doesn’t want others to notice. - Social gathering of teenage friends: Teenage avatars are similar to avatars in other cases with one difference. Teenagers are sensitive to their appearance and may not like some of their own facial and body language expression habits. When using avatars, a teenager can decide to filter out one or more facial or body language expressions and optionally replace them with another expression. A potential user experience is described as a user story: User A is working in the engineering office. She has just completed here latest design and wants to demonstrate it to User B who is working in the factory. She puts on her HMD and haptic gloves and logs in to her company's office in the metaverse using her credentials. After authentication, she is now in the virtual office space and she rings User B, whose video appears on a virtual display. User B can see User A's avatar on his phone's display. User A then asks him to join her in the virtual office to show him what she has been working on and get his feedback. User B puts on his HMD and gloves and joins her. Both User A and User B can now see each other's avatar in the virtual office, where the avatars are seated facing each other across a table. User A loads her design from the virtual panel in front of her and a 3D model of the design suddenly appears on the table. User B wants to take a closer look at the model, so he leans towards it and picks it up. He is now able to feel the surface of the model and inspect it from different angles and he identifies an issue with one of the parts. They need a third opinion, and they contact Layla from another department, who specializes in this particular function. Layla is only able to join the call her AR glasses and headphones, which are tethered to her phone. Layla joins their meeting, and her avatar appears to User A and User B in the virtual office. While she is unable to see the complete virtual office. The avatars of User A and User B, along with the 3D model of User A's deign, are overlayed on her glasses display.

Categorization

Preconditions

Potential Requirements

The following requirements apply in addition to relevant requirements of Use Case 1: - Generating and maintaining a shared digital virtual environment. - User identity management and data security. - Interoperable representation of avatars - The ability to access and load pre-generated digital representations. - Capturing and sending user pose and motion information uplink to the network. - Visual coding and transmission of avatars. - Transmission and potentially coding of motion (animation) data. - The ability to encode and transport sensory information to and from the UE devices, possibly attached to the user’s body - Synchronizing multiple data flows from multiple UEs associated with one user. - QoS requirements that need to be fulfilled in order for the users’ QoE to be satisfactory. - Delivering secure avatar expressions end-to-end for a certain audience - Capability for filtering certain avatar expressions and/or replacing them at the network for some audience - Support the capability of filtering or replacing a selected subset of face and body expressions when generating the avatar animation.

Feasibility

## 5.3	UC3: Multi-user Gaming

TR22.856 Reference Use Case(s)

5.6 Use Case on Mobile Metaverse for Immersive Gaming and Live Shows

Description:

Avatars allow players to express their individuality, represent themselves in virtual worlds, and engage in multiplayer interactions. They enhance the visual aspect of gaming and create a more immersive and engaging player experience. A few examples are listed here below: - Player Representation: Avatars allow players to create digital representations of themselves within games. Players can customize their avatars' appearance, clothing, and accessories to reflect their preferences and identities. Avatars serve as the virtual embodiment of players, enabling them to navigate and interact within the game world. - Immersive Gaming Experience: Avatars enhance immersion in gaming by providing a visual representation of the player within the game environment. Players can see their avatars perform actions, movements, and interactions in real-time. Avatars contribute to a sense of presence and agency, making the gaming experience more immersive and engaging. - Personalization and Progression: Avatars can be customized and upgraded as players progress in games. Players can unlock new customization options, abilities, or items for their avatars as they achieve milestones or complete in-game challenges. Avatars evolve alongside players, reflecting their progress and accomplishments. - Competitive E-sports: In e-sports, avatars are used to represent players or teams in competitive matches. Players control their avatars during tournaments or matches, showcasing their skills and strategies. Avatars serve as the visual representation of players' actions, making e-sports broadcasts more engaging and visually appealing. - Virtual Reality (VR) Gaming: In VR gaming, avatars play a crucial role in providing a virtual presence for players. Avatars represent players within the virtual environment, mimicking their movements and actions. VR avatars enhance the feeling of immersion and embodiment, making the VR gaming experience more realistic and interactive. In this use case, a metaverse mobile gaming service is deployed in the cloud or on an edge server for providing an immersive gaming experience where the players can interact with each other over the 5GS. Players are represented by their avatars and are able to communicate with each other whether they are in close proximity or not. Moreover, the players can interact with each other as well as with other objects in the game's virtual world. This interaction may also wearables that the users have on their hands and bodies which provide haptic feedback in response to actions and interactions that the users are part of. In some gaming scenarios, other participants may also be allowed as spectators where their avatars are rendered as members of the audience in the virtual environment of the game. Different rights may be granted to the avatars of the participants based on their roles (e.g., player, audience, etc.). The terminals of the participants are equipped with sensors and cameras and are connected to control devices to capture their movements and interactions. The captured sensing data are sent to the cloud or the edge server which use this information for coding and rendering to generate the digital representations (avatars). Some avatars may not be user controlled but they are software agents controlled by the game engine. A potential user experience is described as a user story: User B logs in to the metaverse from her living room using her HMD and starts an online shooting game. She is using the controllers connected wirelessly to the HMD to play the game and she is wearing a haptics body suit. A session is established between her HMD and the game server in the network, and she can now select the game mode. She selects a multi-player mode and invites User C and User D to join her. User C and User D are using AR glasses tethered to their phones to join the game and are able to see User B's, as well as each other's, avatar augmented to their surroundings. They opt for using hand gestures to play the game. A fourth avatar, User A, controlled by the game engine appears in each of the player's displays. User B teams with User C while User D and User A are assigned as an opposing team. At some point during play, User B senses a vibration in her body suit and she realizes that her avatar was hit in the game.

Categorization

Preconditions

Potential Requirements

In addition to relevant requirements from the previous use cases, the following requirements shall apply: - Sufficient access controls to ensure age-appropriate content. - Support the transmission of uplink sensor data (raw or processed, e.g., pose, animation data, actions, etc. and downlink feedback information with acceptable packet delay and bandwidth for real-time interaction. - The ability to obtain the location and gestures of the players with reasonable 3D positioning accuracy. - Low latency for interaction and motion data transmission and for rendering. - Minimizing the asynchrony between different modalities (audio, visual, and tactile) to maintain good QoE. The synchronization thresholds are described in [Ref. 49 in TR 22.856] and are listed below: - audio-tactile stimuli: 12 ms when the audio comes first and 25 ms when the tactile comes first to be perceived as being synchronous. - visual-tactile stimuli: 30 ms when the video comes first and 20 ms when the tactile comes first to be perceived as being synchronous. - audio-visual stimuli: 20 ms when the audio comes first and 20 ms when the video comes first to be perceived as being synchronous. Note: The obtained results vary, depending on the kind of stimuli, biasing effects of stimulus range, the psychometric methods employed, etc. - KPI requirements related to the potential requirements [1]: - end-to-end latency: [5~20] ms - service bit rate (user experienced data rate): [1~1000] Mbit/s - positioning accuracy: < 1 m

Feasibility

## 5.4	UC4: Avatar Generation, Storage, and Access

TR22.856 Reference Use Case(s)

5.13 Use Case on Digital Asset Container Information Access and Certification 5.15 Use Case on Access to Avatars 5.16 Use Case on Virtual Store in a Mobile Metaverse Marketplace

Description:

For avatar services, users need a digital representation to represent themselves. Digital representations may be generated by the service provider, and can be created using an app installed on the user's device or by functions provided by the 5G network. When a digital representation is generated on a user's device, it may be pre-uploaded and stored in the network or transmitted to the other party via the network before the start of avatar service. Users may update the digital representation they have stored in the network or transmitted to others. User A wants to engage in an AR avatar service/experience with User B and C in User A's living room. To do this, User A installed an avatar service app a few days ago and followed the app's instructions to capture videos of neutral expressions, smiling expressions, and reading scripts to create User A's digital representation. Upon pre-checking before using the avatar service, User A confirmed that the digital representation could perfectly represent User A's facial expressions and body gestures. The avatar service app recommended pre-uploading the digital representation, taking its size into account, and User A accepted this recommendation. User A may generate several versions of digital representations for different purposes of usage, then upload them with different names, tags, roles or other attributes for future access. User A initiated an avatar service use case (e.g. UC 1, 2, or 3) with User B and C, both of whom also accepted and joined the service, and soon appeared in User A's living room. If multiple avatars were pre-uploaded, User A may choose one of the avatars that fit the target avatar service by searching (or browsing) based on associated attributes of the avatars from the repository. During the avatar service, User A noticed some awkwardness in one of User A's hands and instructed the app to update that part. The app captured the hand again, and the network updated the digital representation in real-time, providing the updated digital representation to User A, B, and C simultaneously. In some scenarios, such as business activities, authorization of avatar usage rights is needed for commercial or other purposes. Therefore, the 5G system needs to support management and authorization of avatar usage rights. The owner of the avatar is expected to be responsible for the speech and behavior of his/her avatar. An individual or an enterprise has to be authorized by the owner of an avatar before using the avatar especially in business activities. Company A registers with the MNO B as an enterprise customer, while User C registers as an individual customer of MNO B. MNO B assigns IDs for Company A and User C respectively. User C registers her personal avatar with MNO B. The avatar is lifelike and mapped to User C’s ID in real world. So MNO B identifies and stores the avatar and its ID, also associates with the avatar’s owner, User C’s ID. Company A sends a request for the usage rights of User C’s avatar that is managed by MNO B. MNO B sends a request to User C’s to ask for authorization of the avatar to be used in the mobile metaverse services. After being confirmed by User C, Company A’s usage rights of avatar has been authorized. MNO B updates the system with the information that Company A has the usage rights of User C’s avatar.

Categorization

Preconditions

Potential Requirements

In addition to relevant requirements from previous use cases, the following requirements shall apply: From SA1: [PR 5.24.6-1] Subject to regulatory requirements, user’s consent and operator’s policy, the 5G system shall support mechanisms to identify an avatar and associate the avatar with a subscriber (i.e. the owner of the avatar). [PR 5.24.6-2] Subject to regulatory requirements, user’s consent and operator’s policy, the 5G system shall be able to authorize the avatar to be used in mobile metaverse services. [PR 5.24.6-3] Subject to regulatory requirements, user’s consent and operator’s policy, the 5G system shall provide time-bound authorization services for an avatar to be used in mobile metaverse services. [PR 5.24.6-4] Subject to regulatory requirements, user consent and operator’s policy, the 5G system shall be able to support mechanisms to manage the authorization information about the use of an avatar in mobile metaverse services (e.g. the applied time-bound authorization services, the authorized users). [PR 5.24.6-5] Subject to regulatory requirements, user consent and operator’s policy, the 5G system shall be able to identify the subscriber who has the right to use an avatar in mobile metaverse services. Re-written - Support the generation, update, and animation (by the UE and/or network media functions) of a user’s digital representation(s) captured from the user devices (e.g., RGB or RGB-D video/images, LiDAR, etc.), subject to user consent, operator policy, and regulatory requirements - Support for avatar metadata that allows for parental control and user access and management. - Support discovery and negotiation to determine the availability of the network functions supporting the media format(s) and media processes related to avatar processing (e.g., generation, storage, update, animation, and AI-based animation generation, etc.). - Support the upload, storage, search, transmission, and update of a user's digital representation while ensuring the consistency of the components of the digital representation, subject to user consent, operator policy, and regulatory requirements. - Support avatar identification and mapping to a subscriber. - Support management and authorization of avatar usage right. - The ability to identify the subscriber who has the right to use an avatar.

Feasibility

## 5.5	UC5:	Artificial Intelligence-Based Avatar

TR22.856 Reference Use Case(s)

5.12 Use Cases on Virtual humans in metaverse 5.17 Use Cases on Work delegation to autonomous virtual alter ego

Description:

An AI avatar is a digital character powered by artificial intelligence, which lives in a virtual setting, like a game, social network or online world. More frequently avatars are designed as human-like bots that can be controlled by real human using AI technologies and can easily engage with real humans and maintain relationship - to varying degrees - with them. This use case proposes a communication with an autonomous virtual alter ego, which is an AI-based digital representation acting autonomously on behalf of a user herself/himself in the mobile metaverse services. For example, user's autonomous virtual alter ego autonomously sends a mail to clients on user’s behalf. Also, the alter ego can autonomously communicate with the user, other physical users, and other alter ego by using the network capabilities based on the user’s 3GPP subscription. Therefore, the use of network by the alter ego has to be captured correctly by the network from charging point of view. NOTE: The term "autonomous virtual alter ego" means an AI-based digital representation behaving autonomously on behalf of a user herself/himself in the mobile metaverse services. All the experience and knowledge performed both in physical world and metaverse will be shared between the alter ego and its user, thus creating more than double the opportunities to play multiple roles simultaneously. This autonomous virtual alter ego concept aims to improve emotional well–being, health, and life satisfaction by enabling users to perceive many opportunities in life, such as balancing work and family and participating in many communities simultaneously. A potential user experience is described as a user story: User A faces a scheduling conflict between an important F2F client meeting and an internal web meeting. To tackle this problem, he connects to the alter ego application via his UE and asks his virtual alter ego (e.g., AI avatar) to participate in the internal web meeting on his behalf. The alter ego first checks and evaluates whether the network resources on 5GS and computing resources on the alter ego server are sufficient to execute the requested tasks. If deemed feasible, the enabler server proceeds to train the alter ego and execute the agree-upon tasks. Otherwise, it proposes alternative task examples to User A, based on its current resources. The alter ego accesses the web meeting server and the necessary meeting information via internet or 5GS by receiving the permission from User A and web meeting server. During the meeting, the alter ego interacts with other attendees, including humans and other virtual alter egos, by explaining, asking questions, or making comments. User A may choose to monitor the meeting participated by his alter ego, and instruct (in the form of voice, text, gesture, etc.) the alter ego to perform certain actions on important issues (e.g. raising hands for voting or commenting). After the meeting, the alter ego autonomously reports back to User A, allowing User A to provide feedback and new instructions if necessary. Additionally, the charging information is collected and linked to user’s charging information to charge the user subscribing the alter ego service.

Categorization

Preconditions

Potential Requirements

In addition to relevant requirements from previous use cases, the following requirements shall apply: - AI/ML model transfer in frameworks documented in TR 23.700-80 [7]. - Securely registering, storing and updating the AI-based digital representation (e.g., AI avatar) for a user. - Assisting the authentication of a third party to use subscriber’s digital representation (e.g., AI avatar) and to access multimedia communication services on behalf of the subscriber. - Providing storage space that a user can use to store application specific data and information about himself in the network. - Supporting transmitting multi-modal instructions to the AI-based digital representation (e.g., AI avatar) for real-time interaction. - Collecting charging information associated with communication involving a digital representation associated with the subscriber.

Feasibility

## 5.6	List of Potential Requirements

# 6	Avatar Representation and Animation Formats

## 6.1	General

## Avatar representation and animation formats define the structural and behavioral characteristics of avatars, ensuring consistency, realism, and interoperability across various services and applications. This section outlines the most relevant existing standardized and proprietary formats for representing the customized/personalized avatars of a user. It also examines animation methodologies, such as blend shape and skeletal animation, morph target animation, procedural motion generation, as well as NN-based animations. The documented formats in this clause are meant to assist with the selection of an appropriate avatar representation format for avatar communications in 3GPP.

## 6.2	Common Terminology

A base avatar is a collection of personalized digital assets that represent the user as a 2D or 3D animatable model. The base avatar may also contain other digital assets associated with the user’s representation, such as digital garments and accessories.

## The animation of a base avatar is based on animation streams that are received from a tracking framework or generated from A/V and other sensor streams.

## 6.3	Existing Avatar Representation Formats

### 6.3.1	MPEG Reference Avatar

#### 6.3.1.1	Body Model

##### 6.3.1.1.1	Overview

The MPEG-I Scene Description reference body avatar (Figure 1) comes with its own mesh topology modeled as a female (right side) or as a male body (left side). The body base mesh is composed of three levels of detail (detailed in Section 6.2.1.1.2). As shown in Figure 1, the density of vertex is not the same for the body and the face, because the later requires more accuracy for realism.

Figure 1: Full body mesh topology (right : female body, left : male body). The gender-neutral face is zoomed for better visualization.

##### 6.3.1.1.2	Levels of Detail

The MPEG-SD reference body avatar is available in three levels of detail (mesh resolutions): low, medium and high (see Figure 2). This ensures that the mesh topology is supported on different runtime platforms (e.g., mobile phone, PC, HMD, etc.). The characteristics of each of these levels are as follows:

- High Resolution (referred to as Morgan_HD)

  - 53,695 vertices and 56,496 quad faces (106,952 triangular faces).

- Medium Resolution (referred to as Morgan_MD).

  - 13,225 vertices and 13,196 quad faces (26,356 triangular faces)

  - Corresponds to 25% of the HD mesh.

- Low Resolution (referred to as Morgan_SD)

  - 5,366 vertices and 5,300 quad faces (10,571 triangular faces).

  - Corresponds to 90% of the HD mesh,

The mesh decimation ensures that vertices positions are the same from a lower level to a higher level. Meaning that all vertices of the SD mesh are all on the MD mesh and all vertices of the MD mesh are on the HD mesh.

Figure 2: Levels of detail of Morgan - From left to right: High (50k), Medium (12k) and Small (5k).

##### 6.3.1.1.3	Model Body Semantics

The body is divided into 47 semantical body regions areas allowing to directly refer to them for specific use cases and applications. Figures 3, 4, and 5 describe these 47 areas for the front, back, and head body parts. The corresponding semantics and vertex coordinates are given by Figure 1 in [3], which provides the correspondence between the semantical regions and the geometric properties of the body model. The vertices and faces are in-order to facilitate interpretation. The example provided corresponds to the "High Resolution" level (Morgan_HD). This allows easy access to parts of the body by any applications, even without knowing the model.

Figure 3: Front view of body semantic areas.

Figure 4 Back view of body semantic areas.

Figure 5: Front-view of the head. Top line: segmentation areas, bottom line: wireframe areas. (Left) complete head view with the face (white), left eye (red) and right eye (olive green) - (Middle) internal view without the face with the mouth bag (blue) - (Right) view without the face and the mouth bag - upper jaw (light green) and lower jaw (purple).

##### 6.3.1.1.4	Base UV Maps

The Morgan model contains multiple UV maps coordinates for all geometric body parts and all LODs. This is based on U-Dimension Mapping (UDIM), which is an enhancement to the UV mapping and texturing workflow that allows to split the UV map into several tiles, each with a resolution adapted to the corresponding region of the mesh geometry. The concept is illustrated in Figure 7 in [3] accompanied by the labelling table below the figure. Each UV tile represents a different texture image with its own UV coordinates.

##### 6.3.1.1.5	Model Skeleton

For the purpose of re-animation and manipulation of Morgan, it is necessary to include skeletal and rig information. Therefore, skeleton joints are also provided in their usual form, i.e., the joints are identified by a naming convention and follow a hierarchical representation to ease manipulation and animation generation techniques. Figure 6 illustrates the complete skeleton hierarchy and the naming convention adopted for Morgan. For simplification and general compatibility with other conventions, Table 1 provides a simplified version of the full skeleton model. The full hierarchical skeletal joint description can be found in Table 2 in [3].

Figure 6: Position of the 63 joints of Morgan

Table 1 Morgan’s Simplified and Simplistic skeleton joints nomenclature.

| Simplified Body Hierarchy (24 joints) | Simplistic Body Hierarchy (15 joints) |
| --- | --- |
| Hips — Spine — — UpperChest — — — Shoulder_Left — — — — LowerArm_Left — — — — — Hand_Left — — — — — — ProximalThumb_Left — — — Shoulder_Right — — — — LowerArm_Right — — — — — Hand_Right — — — — — — ProximalThumb_Right — — — — Neck — — — — — Head — — — — — — Eye_Left — — — — — — Eye_Right — — — — — — Jaw — UpperLeg_Left — — LowerLeg_Left — — — Foot_Left — — — — Toes_Left — UpperLeg_Right — — LowerLeg_Right — — — Foot_Right — — — — Toes_Right | Hips — UpperChest — — Shoulder_Left — — — LowerArm_Left — — — — Hand_Left — — Shoulder_Right — — — LowerArm_Right — — — — Hand_Right — — — Head — UpperLeg_Left — — LowerLeg_Left — — — Foot_Left — UpperLeg_Right — — LowerLeg_Right — — — Foot_Right |

##### 6.3.1.1.6	Skinning

Morgan’s skeleton and full body mesh are linked with skinning weights enabling to animate it using skeleton animation. These skinning weights have been obtained using a two-steps process: automatic linear binding algorithm based on closest distance followed by manual painting corrections by a professional artist. As animation requires highly specific motion types, these weights remain fully modifiable.

#### 6.3.1.2	Face Model

##### 6.3.1.2.1	Overview

Morgan’s face model consists of a base topology and a gender-neutral head morphology. Figure 7 illustrates the geometric composition of the head and face model. The neutral morphology was obtained from a combination of high-resolution face scans dataset containing male and female subjects registered to a reference mesh topology. In the topology depicted in Figure 7, the head mesh is composed of 36, 584 vertices and 36, 362 quad faces, and 72, 784 triangular faces.

Figure 7 Morgan’s face (right to left: front, back, and right profile).

Figure 8 illustrates the four internal parts of the head model, as follows:

- a mouth bag (Figure 8a)

- an upper (Figure 8b) and lower jaw (Figure 8c)

- two eyeballs (Figure 8d)

Figure 8: Morgan’s face details ((a) mouth bag, (b) upper jaw, (c) lower jaw, (d) eyeballs).

Topological details of these specific parts can be found in Table 1 in [3].

##### 6.3.1.2.2 	Face Blend Shapes

The Morgan model provides a collection of facial blend shapes, as listed in Table 4 in [3]. The facial expression shape naming convention is similar but not identical to the FACS naming convention defined and used in anatomy to classify human facial motions [Ekman et al. 1980]. The shapes are separated into Left (“_L”) and Right (“_R”) components. And sometimes these last components have been split into (“_1”) and (“_2”) sub-components to increase the precision.

##### 6.3.1.2.3	Facial Landmarks

To facilitate the registration of Morgan’s face against other facial meshes, or the addition of headwear or glasses to the avatar model, a set of 68 semantic facial landmarks is also provided, as illustrated in Figure 9. These landmarks are also useful for retargeting techniques, or to anchor Morgan’s face to a facial performance tracker solution.

Figure 9: xx

Besides the set of 68 facial landmarks provided in MOGAN, the following sets of facial landmarks may also be used for different levels of implantation complexities:

- 21 points: The definition of 21 points is based on the AFLW (Annotated Facial Landmarks in the wild) dataset. Detailed information on AFLW and a schematic figure can be found in [4].

- 29 points: The definition of 29 points is based on the LFPW（Labeled Face Parts in the Wild）dataset. Detailed information on LFPW and a schematic figure can be found in [5].

- 98 points: The definition of 29 points is based on the WFLW (Wider Facial Landmark in the Wild) dataset. Detailed information on WFLW and a schematic figure can be found in [6].

##### 6.3.1.2.4	Teeth Model

The facial rig of Morgan includes an upper and a lower jaw, as illustrated in Figure 12 in [3]. The lower jaw is composed of 10, 440 vertices and 10, 380 quad faces (20, 760 triangular faces). The upper jaw is composed of 11, 107 vertices, 11, 040 quad faces and 22, 080 triangular faces. They both contain separate UV coordinates and associated textures.

##### 6.3.1.2.5 	Eye Model

Morgan comes with a complete eye model depicted in Figure 10. The eye model follows an anatomical eye model and is composed of three elements: the sclera, the iris, and the lens. The polygon count is as follows:

-	 lens: 201 vertices and 200 quad faces (380 triangular faces)

-	 iris: 928 vertices and 896 quad faces (1792 triangular faces)

-	 sclera: 290 vertices and 288 quad faces (576 triangular faces)

Figure 10: Morgan’s eyes model.

The eye model of Morgan comes with two blend shapes for each eye, as follows:

-	Pupil_Dilatation: controls the aperture of the pupil of the eye.

-	Sclera_Flatness: controls the degree of flatness of the cornea.

### 6.3.2	Skinning and Animation in glTF 2.0

Skinning is a technique that is widely used to animate a 3D model by deforming its mesh based on the movements of an underlying skeleton. This allows for realistic animations of characters and other complex models.

To enable skinning, a skeleton is required. The skeleton consists of a root node and a hierarchical set of joints that define the structure and movement of the model. Each joint influences one or more vertices of the mesh. In glTF 2.0, the joints are represented as nodes which are associated with their inverse bind matrices. The inverse bind matrix of a joint defines the transform that is applied to each vertex of the associated mesh to bring the vertices to the joint’s space. This is in preparation of applying the world transform on the joint and its vertices.

The information about skeleton, joints, and their inverse bind matrices is provided as part of the skin object.

The mesh itself may provide a set of joints that may influence the position of each vertex. A maximum of 4 joints are allowed for any single vertex. The amount of influence is determined by weight associated with each of these 4 joints. The joints and weights are referenced through the attributes JOINTS_i and WEIGHTS_i, where is i can range from 0 to 3.

Animation in glTF 2.0 may be used to animate or retarget the skinned character. Animations in glTF 2.0 are keyframe based and allow for modification of the properties of a node in the graph, such as its transformation or rotation. The modification follows an interpolation method and allows the values to transition from their initial to their target values over time. Animation may be used to animate the skeleton of the character by transforming the joint’s positions over time to deform the mesh.

The following is an example asset that is released by Khronos to demonstrate the capabilities of skinning and animation.

Retargeting can be performed by converting the detected body pose into an animation stream for the skeleton of the character.

### 6.3.3	Avatar Support in MPEG-I Scene Description

MPEG-I Scene Description has introduced a new extension to glTF 2.0, the MPEG_node_avatar, to indicate that a particular node is the root/skeleton node for a humanoid Avatar character. The extension allows for the node content to be filled at runtime after a 3D reconstruction step is performed. The extension allows for signaling the Avatar representation format and associating the different joints of the Avatar with their joint identifier and hierarchy. The skeleton node is assumed to contain a mesh reference, that points into a dynamic source, which provides the animation data for the 3D reconstruction of the animated Avatar.

The following is a description of the properties of the MPEG_node_avatar extension:

| Name | Type | Usage | Default | Description |
| --- | --- | --- | --- | --- |
| type | string | M |  | The type of the avatar representation is provided as a URN that uniquely identified the avatar representation scheme. The avatar representation scheme defines the format of all components that are used to reconstruct and animate the avatar. The reference MPEG avatar URN is defined in section 8.3.3. |
| mappings | array(Mapping) | M |  | The mapping between child nodes and their associated avatar path. Note that the corresponding path for a parent node shall be a prefix of the path of its child nodes. |

As mentioned earlier, the type is a URN identifier that uniquely identifies the Avatar representation format. The mappings element is crucial for enabling interactivity applications by correctly segmenting the 3D reconstructed avatar.

In the MPEG-I Scene Description, the avatar is signaled at the node level by the definition of the MPEG_node_avatar extension. Based on the glTF 2.0 and MPEG-I Scene Description, any node may have child node(s). Each node (including any child nodes of the node) has properties and may have an optional extension property that can be used by this node. The avatar node and its child nodes can reuse all the properties defined by glTF 2.0 and the extension properties defined by the MPEG-I Scene Description.

As described in clauses 6.3.1, 6.3.2, and 6.3.4, an avatar representation format would include a geometrical model and all associated data (e.g., blend shapes, skeleton, textures, etc.). Some associated data is the data of an avatar property.

Some avatar properties can be changed dynamically to create some special effects for the avatar. One example is the texture of the avatar. The texture is commonly used in the avatar representation for some parts of the avatar (e.g., body). The texture can be mapped from an image as described in glTF 2.0 or from a video stream as described in the MPEG-I Scene Description. During the skinning and animation, if the texture of an avatar part is mapped from a video, the texture dynamically changes with the video frames.

### 6.3.4	MPEG Avatar Representation Format

The MPEG WG03 (Systems) workgroup is working on a new standard for defining a new avatar representation format. The objectives of this new standard and the envisaged representation format can be found in the publicly available output document number WG03N1316 [7].

The scope of the work started in MPEG is to develop an interchange representation format for computer generated avatars and associated containers, and an animation stream format to represent the avatar dynamics and timed-based information. This avatar representation format would include a geometrical model and all associated data (e.g., blendshapes, skeleton, normals, textures, maps, metadata, etc.), as well as a streamable format for all the dynamics associated to this model (e.g., animation parameters, tracking information, contextual data, etc.). Interoperability between existing models and formats and the MPEG avatar representation format is also taken into consideration in the development of this new representation format.

The planned timeline for the first phase of this MPEG standard is shown in Figure 11.

Figure 11 - Phase 1 timeline for MPEG avatar representation format.

The identified requirements and their priorities for Phase 1 of the MPEG avatar representation format standardization work are documented in [7] and are shown in the following table, where the priorities are defined as follows:

-	Low – should be considered once the higher-level requirements are fulfilled or partially acceptable.

-	Medium – should be considered once higher-level requirements partially acceptable.

-	High – should be the first to be considered for the technical solution.

Table 2 - Requirements for Phase 1 of the MPEG Avatar Representation Format

| Category | Description | Priority |
| --- | --- | --- |
| Avatar Representation | A suitable exchange format for conversion between avatar representation formats in market | High |
|  | Representation format includes description of body, skeletal, etc. | High |
|  | Representation format does not prevent DRM protection | Medium |
|  | Mesh-based format for representation and animation | High |
|  | Integration into scene description | Medium |
|  | Signal coding format | High |
| General | Semantic and signal representation - Semantic - high level features - Signal - low level features | High |
| Geometry | Multiple levels of detail | High |
|  | Reference known 3D models | High |
| Interaction | Avatar-avatar, user-avatar, avatar-scene interactions | Low |
|  | Integrates with existing interaction frameworks | Low |
| Animation and control | Facial and body animation | High |
|  | Storage and replay of animation streams | Low |
|  | Efficient animation through: - External sensors - Pre-defined actions | - High - Low-Medium |
|  | Animation through - Pre-defined actions - External sensors | - Low-Medium - High |
| Transport | Delay sensitive animation streams | High |
|  | Partial transport of base avatar | High |
|  | Uniquely identifiable avatar representation and animation streams | High |
|  | MPEG format as fallback | High |
|  | Reuse of animation streams for different formats (potentially with loss) | High |
| Storage | Base avatar stored in single file | High |
|  | Partial access | High |
|  | Independent component compression | High |
|  | Independent component encryption | High |
|  | Different levels of detail | High |
|  | Entry point to describe components and relationships | High |
|  | Identify format for interoperability | High |
| Privacy/Security | Protection of authenticity and association with user | Medium |

The aspects that will be technically addressed by MPEG related to this JSON avatar format include what the JSON avatar format should contain as information related to an avatar and serves as a baseline for the continuation of the work to add more features in the future.

The representation baseline format is expected to contain the following data:

- High-level avatar information:

- Name

- Identifier

- Age

- Gender

- Representation data:

- reference template

- mesh: one or more geometries (LoDs).

- body mappings: a list of string with associated geometries to identify the body parts

- skeletons: one or more skeleton skins

- controllers: one or more animation controllers (inc. facial blend shapes, joints, morph targets)

- Additional information:

- Model mappings: one or more mapping function (reference template to B)

- Animations: a set of predefined animations

- Processing functions

- Avatar gaze

- Parametric textures

- Hair

- Accessories

## 6.4	Animation Approaches and Formats

### 6.4.1	Morph target animation

Morph target animation, also known as shape key animation or blend shape animation, is a technique commonly used in computer graphics and 3D modeling to create smooth and realistic deformations of 3D models, including avatars as illustrated in [ref: https://docs.unrealengine.com/udk/Three/MorphTargets.html]. It is especially useful for animating facial expressions, character deformations, and other shape changes.

Morph Target animation relies on the following steps:

-	Shape Keys/Targets: The first step in creating morph target animation is to define a set of shape keys or targets. Each target represents a specific deformation or shape change to be animated. For example, when animating a facial expression, targets may be a smiling face, a frowning face, and a neutral face.

-	Base Mesh: Considering for example a Mesh representation of the avatar, the based mesh is the default and neutral state of the avatar. This base mesh is the starting point for all animations.

-	Deformation Calculation: To create an animation, one or more of the target shapes are interpolated with the base mesh. The interpolation factor determines the influence of each target shape on the base mesh. Examples of interpolation include LERP (linear interpolation), SLERP (Spherical linear interpolation), facial rigging and bone-based animation…

-	Animation Control: The amount of deformation applied to the base mesh may be controlled by adjusting the interpolation factor. For example, the use of weighting factor on a skeletal rig will influence how much the body of the avatar moves.

-	Keyframes: Keyframes may be created at specific points in time to control the progression of the animation. Each keyframe specifies the interpolation factor for the target shapes.

-	Real-Time Control: On a UE with 3D graphics capabilities, morph target animations may be controlled in real-time. This is intended to be used for interactive applications, such as video games, where the player input or some server-based animations determine the deformation of the avatar.

Morph target animation is particularly well-suited for animating facial expressions, lip syncing, and other detailed deformations, as it allows for precise control over the shape of the avatar model. It is commonly used in the animation and game development industries, and it is a key component in creating lifelike avatars.

## 6.5	Existing 2D Avatar Representations

### 6.5.1	Introduction

A 2D avatar (or a 2D digital human) is a graphic representation of a digital human in the two-dimensional space. This representation has no depth and is usually created and viewed from a fixed point of view.  See the relevant ITU-T definitions quoted below for the ease of reference:

- Digital Human: A computer application that integrates the technologies of computer graphics, computer vision, intelligent speech and natural language processing, which could be used for digital content generation and human-computer interaction to help improve content production efficiency and user experience. [11]

- 2D Digital Human: A digital human whose figure is a planar image and whose graphic content only contains information about horizontal and vertical dimensions. [12]

In general, the 2D avatar may share the common framework and procedure with the 3D avatar, while it has some unique features as below:

- AI-based modelling: It doesn't require the building of a 3D mesh topology as the base model. Instead, a 2D avatar base model may be generated through the AI training against a Deep Neural Network (DNN).

- Direct animation: A 2D avatar may be driven directly from the users’ input of video, audio or text. It doesn’t need to first derive the animation data from the input media, nor to have the 3D rendering process later.

- Flexible deployment: A 2D avatar may be deployed over either NPU or GPU resources for the model generation (training) and animation (inference) due to its nature of AI-based modelling.

Since a 2D avatar doesn't have a 3D base model that may generate significant body movements and provide free viewports, it may only provide limited viewport (usually fixed) and gestures of the character. This makes it hard to support motional use cases like VR games. Nevertheless, the 2D avatar may be quite suitable for the cases where the position and posture of the avatar is relatively fixed such as RTC.

Detailed information about typical 2D avatar use cases and technical practices is provided in the following clauses.

### 6.5.2	2D Avatar Use Cases

The 2D avatar is suitable for planar display and simple interaction scenarios which does not require multi-view experience. Typical use cases are the following:

Figure 6.5.2-1. 2D Avatar scenarios (top-left: Avatar call, top-right: Virtual anchor, bottom-left: Virtual customer service, bottom-right: Social media)

-	Avatar call: The 2D avatar may be introduced to upgrade the user experience from a traditional voice or video call. The 2D avatar may be animated by the user’s voice or captured video in real-time and displayed on the screen of the peer user. This service may increase the fun of the real-time communication and protect the user’s privacy in the meantime. An example for avatar call is shown at the top-left of Figure 6.5.2-1.

-	Virtual anchor/E-commerce: 2D Avatar may be used as virtual anchor for news broadcasting or shopping/touring guide to introduce product items or place of interests. The 2D avatar may be animated by the user’s voice, video or text scripts, which may be fed in real-time or pre-recorded. This service may enable 7*24 online service while relieving the burden of human labours. An example for 2D avatar virtual anchor is shown at the top-right of Figure 6.5.2-1.

-	Virtual customer service: When a user dials the customer service number, the 2D avatar of customer service agent may be displayed on the user’s screen instead of the real agent person. The 2D avatar may provide simple interaction following instructions of a human agent or driven by AI. This service may improve the friendliness of the customer service and provide the possibility of 7*24 online service when the human agent is not available. An example for 2D avatar customer service is shown at the bottom-left of Figure 6.5.2-1.

-	Social media: In social media scenario, cartoon-styled 2D avatars animated by the user's body action or facial expression may be used as the personalized/customized figure in the user generated contents to protect user’s privacy and increase the attractiveness. This service has been widely used to in social media sharing such as short videos. An example of 2D avatar short video is shown at the bottom-right of Figure 6.5.2-1.

### 6.5.3	2D Base Avatar generation

The 2D Base Avatar includes the following components:

-	DNN model: It is a Deep Neural Network model used to generate the animated target avatar through the inference process driven by the user’s input such as audio, video or text [20][21]. The DNN model may be optionally stored in the Avatar Storage and downloaded for avatar animation.

-	Base image/video: It is a 2D image/video in order to specify the target avatar character based on the DNN model. It is stored together with the DNN model in the Avatar Storage and will be downloaded for avatar animation (see details in clause 6.5.4). It may be generated by AIGC tools offline. The base video may help to improve the performance of the avatar animation compared to the base image.

The process of 2D Avatar DNN model generation is to train a DNN model through a supervised learning algorithm. The result model is the DNN model for a 2D Avatar. The training process is depicted in Figure 6.5.3-1.

Figure 6.5.3-1: 2D Avatar DNN model generation process (training)

The training process of a 2D Avatar DNN model takes three inputs as listed in Table 6.5.3-1:

-	Reference image: provides the baseline reference of the 2D Avatar representation in the form of a picture.

-	Ground truth: It is the target output video of the 2D Avatar to be trained against. The reference image should have the same character as in the ground truth video but not exactly the same appearance at the same time. For example, the lip shape or the facial expression may be different between the reference image and the ground truth at a certain time point.

-	Training input: It may be video, audio or text depending on the target use cases (i.e., which type media is going to be used for avatar animation at the inference stage).

The relationship between the reference image, training input, and the ground truth are the following:

- If the training input is audio, the ground truth is the video where the character in the reference image is talking as recorded in the audio [13].

- If the training input is video, the ground truth is the video where the character in the reference image makes the same action [14]. (In this case, the character in the training input is usually different from the ones in the reference image and the ground truth video for the use case of “face swap”.)

- If the training input is text, the ground truth is the video where the character in the reference image is talking the text content [15].

A typical DNN used for the 2D Avatar has the encoder-decoder architecture. The encoder part is used to map the input to the embedding space, and the decoder part is utilized to map the embeddings to the image space so as to generate the target 2D avatar video. A pre-processing function may be introduced to perform cropping, resizing, and normalization on the reference image and the training input data before injecting them into the DNN. It may be implemented inside or outside of the DNN. The loss function defines a measurement of the difference between the generated video and the ground truth video. The output of the loss function is used to perform the backpropagation to update the DNN parameters.

Upon the training process completion, the converged DNN results as the DNN model of the 2D Avatar. The DNN model may be stored in an exchangeable format such as .pth (PyTorch [16]), .onnx (ONNX [17]),  .trt (TensorRT [18]) or mindir (MindSpore [19]) files. In order to support the interoperability of the DNN model between UE and the network, the metadata of the model information (such as model identifier, input/output size, format/framework & version number, processing capabilities) as defined in clause 6.6.2 of [22] may be used.

Considering different levels of generalization capabilities, 2D Avatar DNN models may be categorized in two groups:

-	Robust model: It provides good generalization capability that may support the animation of different target 2D avatar characters (designated by different base images/videos).

-	Personalized model: It is good at the animation of a particular target 2D avatar character (designated by the base image/video of the same avatar character as in the reference image) but not others. A personalized model may be a fine-tuned model from a robust model.

The cost of training a robust model is generally higher than that of a personalized model, since the former takes much more input training data and training time for different avatar characters. Based on the state-of-the-art implementations and typical hardware conditions, a robust model could be trained within a day, while a personalized model could be trained less than one hour.

Table 6.5.3-1: Training input data for 2D Avatar DNN model generation

| Training input data | Description | Requirements |
| --- | --- | --- |
| Reference image | The baseline reference of the 2D avatar character to be trained. | The character in the reference image is the same as the one in the ground truth, while the character’s actions (e.g. mouth movements or facial expressions) are generally different at the same point in time. |
| Training input | The media content (video, audio, or text) to drive the animation of the 2D avatar character out of the DNN. | The actions in the training input are consistent with the ones in the ground truth video (e.g. the syllables of speaking match the mouth shape). |
| Ground truth | The training target of the 2D avatar. | The character in the ground truth video is consistent with the reference image, and the character’s action is consistent with the one in the training input. |

### 6.5.4	2D Avatar animation

The process of 2D Avatar animation is to generate the 2D video representation of the avatar through the inference process of the DNN model driven by the input video, audio or text from the user.

The inference process is depicted in Figure 6.5.4-1.

Figure 6.5.4-1: 2D Avatar animation process (inference)

For different use cases of avatar animation, the inference process may take different input data combinations as listed in Table 6.5.4-1:

-	Base image/video: It is used to specify the target avatar character to be driven. If a robust model is used as the DNN model, the base image/video may be an avatar character different from the reference image used in the DNN model generation phase. If a personalized model is used as the DNN model, the base image/video should use the same avatar character as the reference image.

-	Driving input: It may be the audio or video stream coming from the user during a real-time call. It may also be a text stream converted from the audio signals depending on the input requirement of the DNN model.

As described in clause 6.5.3, the DNN model may be implemented as a robust model or a personalized model. The robust model may be shared for general users and don’t need to be downloaded for animation every time. Instead, different base images/videos should be downloaded in order to support the animation of user specific avatar representations. In the case of the personalized model being used, different DNN models and the corresponding base images/videos will be downloaded for the avatar animation of different users.

The output of the DNN model inference is essentially the animated 2D Avatar, which may be further encapsulated, streamed and played with any existing video codecs (e.g. H.264/H.265) and deliver protocols (e.g. RTP).

Table 6.5.4-1: Possible input data combinations for 2D Avatar animation

| DNN Model | Driving input | Base image/video | Notes |
| --- | --- | --- | --- |
| Robust model | Video | Base image | The base image/video may use a different avatar character from the reference image used in the DNN model generation process. |
|  | Audio | Base image/video |  |
|  | Text | Base image/video |  |
| Personalized model | Video | Base image | The base image/video should use the same avatar character as in the reference image used in the DNN model generation process. |
|  | Audio | Base image/video |  |
|  | Text | Base image/video |  |

# 7	Reference Architecture

The following figure depicts the reference Architecture for Avatar:

Figure 11. Avatar Reference Architecture

The identified Avatar functions are:

-	Avatar Storage: an entity that offers storage of base avatars. This entity may be offered by the 5G System, a 3rd party entity, or the local storage of the user’s devices. The Avatar Storage ensures proper access to the base avatar and any related data, including authorization of avatar usage rights. The Authentication functionality should be able to map and identify the ownership of an avatar.

-	Avatar Animation: depending on the avatar representation format, this entity retrieves the base Avatar, receives representation format-specific animation data streams, and performs the avatar animation to produce the animated avatar that will be used in the rendering process. 
[Note that some animation approaches may not need to rely on the 3D base avatar, instead they directly produce rendered 2D view of the Avatar.]

-	Scene Management: creates and composes the shared 3D scene for all participants. It integrates a description of the user’s Avatar and updates its position and orientation based on the user’s pose, and its properties (e.g. the textures if the texture source is changed or the texture is mapped from a video). The updated scene is shared with all participants.

-	Animation data generation: generating animation data from raw signals. The raw signals may come from cameras, microphones, and specialized motion capturing devices, etc. For example, through the current functional element, the video captured by the camera can be converted into facial feature points, and the audio captured by the microphone can be converted into text, etc.

-	Base Avatar Generation: generates the base avatar from the inputs such as captured video from camera and other sensors information, possibly in conjunction with a reference avatar. Note that this might be done online or offline.

Figure 12 shows several examples of workflows where avatar functions are differently distributed between the sending UE (UE-A), the network, and the receiving UE (UE-B). The type of avatar data delivered from UE-A (highlighted by the blue arrows) is inherently different depending on which avatar functions are performed on the device by UE-A.

Figure 12. Avatar workflows showing avatar functions performed by different entities

The decision to use a certain avatar workflow depends on the requirements of the specific avatar service, as well as the configurations between the sending (UE-A) and receiving (UE-B) UEs, and the network. More specifically:

-	The avatar functions supported by the sending UE (UE-A),

-	The avatar functions supported by the receiving UE (UE-B),

-	Negotiation of the avatar workflow configuration to request certain avatar functions to be performed by the network.

# 8	Avatar Integration into 3GPP Services and Enablers

## 8.1	Introduction

## 8.2	Signalling

## 8.3	Transport

## 8.4	QoS Support

## 8.5	Split Rendering of Avatars

## 8.6	Mapping to IMS-based Services

## 8.7	Mapping to non-IMS-based Services

### 8.7.1	Architecture Mapping

The architecture shown in Figure 12 demonstrates a mapping of the avatar reference architecture described in clause 7 to the General RTC Architecture defined in TS 26.506 and based on collaboration scenario 3 of that specification.

Figure 13 - RTC Architecture for Avatar-based Communication

In this architecture an RTC application for avatar-based communication (represented as “Avatar Application”) interacts with an RTC Client to establish WebRTC-based RTC sessions with a Media AS that supports avatar functions. Note that while the Avatar Storage function is not shown in Figure 13, this function can be part of the UE or the RTC AS. The avatar functions supported by the Media Function include:

-	Base Avatar Generation: the MF may generate the base avatar from captured sensor data on the participant’s UE and made available to the MF.

-	Animation Data Generation: the MF can generate an animation data stream based on the media received from the user that can be applied to the base avatar to animate it.

-	Avatar Animation: the MF can apply an animation stream that is either generated locally in the MF itself or received from the UE to a base avatar to animate it.

-	Scene Management: the MF may create and maintain a scene graph representation of the scene in which the avatars and other media objects may be placed.

NOTE: The location of the Scene Management function is TBD.

### 8.7.2	Call flows

#### 8.7.2.1	Call setup and capability negotiation

Figure 13 demonstrates a call flow diagram for call setup and capability negotiation between the RTC application running on UE1, the MF, and a remote endpoint (UE2).

Figure 14 - Call flow for call setup and capability negotiation

The steps shown in Figure 14 are as follows:

1: Service provisioning and announcement of an RTC service on the network side, in particular between the Media AF (application function) and the Media AP (application provider).

2: The RTC MSH requests configuration information from the RTC AF relating to RTC sessions.

3: The RTC AF provides the requested configuration information to the RTC MSH.

4: The application queries the RTC MSH through the RTC Access Function for the WebRTC Signalling Function information and retrieves this information. In some cases, where the signalling is handled by the RTC Access Function, the RTC Access Function directly queries the RTC MSH for this information.

5: 6: Two or more RTC endpoints exchange signalling information related to the RTC session, such as ICE candidates and SDP offer/answer through the SWAP protocol using the trusted WebRTC Signalling Function in the RTC AS. The SDP offer/answer messages may also be used to negotiate avatar-related capabilities such as an avatar representation format or an animation data format as well as support for data channels.

#### 8.7.2.2	Avatar Media and Metadata Exchange

Figure 15 demonstrates a call flow diagram for the exchange of avatar media and associated metadata between relevant entities in the architecture mapping presented in clause 8.7.1 after call setup has been complete and an RTC session has been established between the MF, the RTC endpoint on UE1, and the remote endpoint (UE2).

Figure 15 - Call flow for Avatar Media and Metadata Exchange

The description of the steps shown in Figure 15 is as follows:

B. Scene Description Retrieval

-	The MF and the participating UEs retrieve scene descriptions, the scene description may be shared by the MF with the UEs, or the UEs may have their own scene descriptions.

C. Scene Description Update

-	A scene update trigger occurs, e.g., if an object is added to or removed from a scene or if spatial information is updated. The update trigger may originate from the MF itself or the UEs. The UEs may update their scene descriptions independently or the MF may generate an updated scene description and share it with the UEs.

D.1 Avatar Acquisition

-	D.1a.1:	The RTC application on UE1 sends captured data needed to generate the base avatar to the Media Function in the RTC AS through one or more RTP streams over the RTC session.

-	D.1a.2:	The MF uses the captured data sent by UE1 to generate the base avatar for the user.

NOTE: 	When the base avatar is generated in the network by the MF, the generated base avatar may be stored in the Avatar Storage for future loading.

-	D.1b.1:	The MF loads the base avatar for UE1, identified by the negotiation step, from the Avatar Storage.

D.2 Avatar Delivery

-	D.2.1:	The MF creates a reliable Data Channel for the delivery of the base avatar to UE2.

-	D.2.2:	The MF creates a reliable Data Channel for the delivery of the base avatar to UE1.

D.3 Animation Data Generation

-	D.3a.1:	UE1 sends source animation data (e.g., sensor data, audio, video, text) to the MF through media or data channels.

-	D.3a.2:	The MF processes the received source data to generate an animation data stream for the session.

-	D.3a.3:	The MF delivers the generated animation data through a media or data channel to UE2.

NOTE:	In network-centric avatar animation data generation, the animation data generated by the MF may also be delivered to UE1.

-	D.3b.1:	UE1 uses data captured by its sensors to generate an animation data stream.

-	D.3b.2:	UE1 sends the generated animation data stream to UE2 through the MF over a media or data channel.

D.4 Avatar Animation

-	D.4a.1:	The MF uses the animation data (generated by the MF itself in step D.3a.2 or received from UE1 in step D.3b.2) to animate the base avatar.

-	D.4a.2:	The MF sends an animated avatar stream to UE1 and UE2.

-	D.4b.1:	UE1 animates the base avatar using the animation data stream generated in step D.3b.1.

-	D.4b.2:	UE1 sends an animated avatar stream directly to UE2 through a media channel.

D.5 Avatar Rendering and Display

- UE2 renders the animated avatar based on the target viewport and pose.

# 9	Security and Privacy

## 9.1	General

The integration of 3D avatars in augmented reality (AR) calls has become increasingly popular. These 3D avatars offer users a way to represent themselves virtually in a more personalized and interactive manner, mimicking the user's facial expressions and body gestures.

The use of 3D avatars in AR calls provides a dynamic and immersive experience, allowing users to engage with others in a virtual environment that can be more engaging and expressive than traditional video calls. These avatars can be customized to resemble the user closely and can be dressed according to the user’s desire.

While this technology presents numerous opportunities for enabling a more immersive user experience, it also brings with it significant risks, particularly around user privacy and security questions. Unfortunately, the very features that make 3D avatars appealing also open the door to potential misuse, particularly in the form of impersonation. When a user creates a 3D avatar, especially one that closely resembles their real-life appearance, this model can potentially be misappropriated by malicious actors.

One of the most significant risks occurs when the 3D avatar base model is exchanged with other participants in the call for animation. Once the assets are downloaded, a malicious user may offer it in the future as their own base avatar model to other participants.

Impersonation can be particularly dangerous in scenarios where 3D avatars are used for authentication or verification purposes. For instance, some platforms may use the uniqueness of a user's avatar as part of a security protocol. If an attacker successfully replicates or steals the avatar, they could bypass these security measures, leading to unauthorized access to accounts or personal information.

## 9.2	Relevant Protection Techniques

### 9.2.1	3D Watermarking for Mesh-based Avatar Protection

Editor’s Note: how this technique can be leveraged for the protection of Avatars in an AR call is FFS.

#### 9.2.1.1	Introduction

Digital watermarking is a key technology for maintaining the integrity, ownership, and authenticity of 3D assets. However the implementation of this technology to mesh-based avatars in real-time communication is for further study. The goal is to embed a message in 3D meshes that can withstand various attacks imperceptibly during the distribution and use of avatars, and reconstruct the message accurately from distorted/undistorted watermarked 3D meshes.

There are four requirements for 3D mesh watermarking:

-		Rubustness: The watermark must be resilient against various attacks such as compression, scaling, and		noise during transmission channels. The robustness of digital watermarking against a specific type of 			attack can be measured by the accuracy of watermark extracted from the attacked mesh.

-		Imperceptibility: The watermark should not visibly change the appearance of the 3D mesh, ensuring that 		the visual quality of the avatars remains unchanged.

-		Efficiency: The watermarking and extraction processes should be computationally efficient, ensuring that 		they do not introduce significant overhead or delay in the use and distribution of the avatars.

-		Capacity: The ability to embed a larger amount of messages into the mesh.

Note: how this technique can be leveraged for the protection of Avatars in an AR call is FFS.

#### 9.2.1.2 	3D Mesh Watermarking Methods

Existing 3D mesh watermarking methods can be categorized into traditional methods and deep learning-based methods. Traditional methods manually design watermark embedding and extraction algorithms in the spatial or transform domain to resist specific types of attacks. However, manually designing a watermarking algorithm for each specific scenario is both time-consuming and labor-intensive, and most of manually designed watermarking algorithms struggle to achieve universal robustness against various attack forms, such as clipping and smoothing.

Deep learning based-methods for 3D mesh watermarking achieved great progress in recent years. Deep learning-based watermarking techniques can optimize watermark embedding and decoding processes by simulating various types of attacks, which effectively enhance the universality of the watermark. It has been demonstrated that deep learning-based methods are effective and robust for copyright traceability of mesh-based digital avatar data files [9, 10, 11]. Figure 16 shows the generic pipeline of deep learning-based 3D mesh watermarking.

Figure 16 Generic Pipeline for Deep Learning-based 3D Mesh Watermarking

The procedures of deep learning-based 3D mesh watermarking are summarized below:

S1. Acquire the original 3D mesh and embedding watermark;

S2. The pre-trained watermark processing system takes the original 3D mesh and embedding watermark as input, and use neural network model(s) simultaneously embed watermark into the geometric and texture parts of the 3D mesh.

S3. The watermarked 3D mesh may be attacked (e.g., transformations, random rotation, nosing, clipping, etc) during avatar transmission or implementation.

S4. The target watermark can be extracted from the attacked watermarked 3D mesh based on either the geometric part or texture part by neural network model(s). The target watermark should be consistent with the input watermark.

#### 9.2.1.3 	Mesh-based Avatar Protection

Mesh is a commonly used avatar representation format. Figure 17 provides the framework for implementing deep learning-based 3D mesh watermarking into mesh-based avatar protection without affecting the usability of digital avatar representations. .

Figure 17 Deep Learning-based 3D Mesh Watermarking for Avatar Protection

Data Pre-processing Module: Pre-process the 3D avatar file to obtain the original 3D mesh from the Avatar Storage entity.

Watermark Embedding Module: The geometric and texture parts of the 3D mesh are simultaneously embedded watermark to generate the watermarked 3D mesh. The geometric watermark embedding network extracts feature from geometric part of the 3D mesh, combining it with input watermark, and reconstructs the watermarked geometric part. The texture watermark embedding network follows the similar processes to reconstruct the watermarked textures.

Attack Simulation Module: During the training process, the attack simulation module simulates various attacks, including transformations, random rotation, nosing, clipping, etc, and applies them to the watermarked mesh during the end-to-end training of the framework, to enhance the robustness of the watermark against universal attacks.

Watermark Extraction Module: Features are extracted separately from the geometric structure and/or texture map of the attacked 3D mesh. The geometry and the texture features are independently fed into the watermark recovery network to obtain target watermark. This means that even if one part of the mesh's geometry or texture is intentionally removed, the complete watermark can still be extracted from the other part, thereby achieving the goal of copyright tracing.

### 9.2.2	Feature Verification

In order to minimize the risk of impersonation by other participants of an AR call that are granted access to the user’s base avatar model, the following measures should be taken:

- DRM protection of the base avatar model and encryption of key components of the base avatar model, such as the blendshapes, the mesh components, key textures, and/or the skin binding information,

- Encrypted storage of user-authenticating information in the base avatar model,

- During an AR call, receivers continuously authenticate the user by matching user-authenticating information from the base avatar model to the same information extracted from live user streams.

NOTE: the security risks of storing user authenticating features as part of the base avatar model are FFS.

One such user-authenticating information are the facial features, which can be stored as part of the user’s base avatar model and encrypted using their private key to prevent tampering or replacement.

During an AR call the receivers access these features by decrypting them using the public security key of the sender. They also periodically run the same facial feature extraction algorithm on key frames from the user’s video stream and compare them to the features in the base avatar model. A strong or prolonged deviation is a strong indicator of an impersonation attempt. The same approach can be used with user’s voice by deploying a voice feature extractor.

Facial features are key points or regions on the face that capture significant information about an individual's appearance, often used in facial recognition, emotion detection, and identity verification systems. These features typically include landmarks such as the eyes, nose, mouth, and the contours of the face, which provide critical data about the geometry and structure of the face. The extraction of these features involves identifying specific points on the face, known as facial landmarks, which can number anywhere from a few dozen to several hundred depending on the complexity of the model. These landmarks represent the spatial coordinates of important facial regions, allowing algorithms to map and analyze the face's shape and expressions. Beyond landmarks, other facial features can include texture patterns, skin tone, and even fine details like wrinkles or freckles, which contribute to the uniqueness of a person's face.

The extraction of facial features is typically performed using algorithms like Active Shape Models (ASM) [19], Active Appearance Models (AAM) [20], or more recently, deep learning-based methods like Convolutional Neural Networks (CNNs) [21]. Traditional methods, such as ASM and AAM, rely on statistical models to learn the shape and appearance of facial features from a training set and then fit these models to new images by adjusting the parameters to match the observed data. CNNs, on the other hand, are trained on large datasets of facial images and automatically learn to identify and extract relevant features by processing the image through multiple layers of convolutions and pooling. These networks can capture both local and global features, allowing them to detect subtle variations in facial expressions or slight changes in orientation. Modern approaches often combine landmark detection with feature extraction in a unified framework, resulting in robust and accurate models that can be used in real-time applications such as facial recognition systems or augmented reality.

The following diagram depicts the procedure:

# 10	Conclusions

Annex <F> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 25/8/2023 | 125 |  |  |  |  | Initial draft of the TR | 0.1.0 |
| 24/10/2023 | post 125 | S4aV230059 |  |  |  | Added Use Case 5 | 0.1.5 |
| 24/10/2023 | post 125 | S4aV230060 |  |  |  | Updated Use Case 4 | 0.1.5 |
| 17/11/2023 | 126 | S4-231877 |  |  |  | MPEG reference avatar | 0.2.0 |
| 17/11/2023 | 126 | S4-232033 |  |  |  | Avatar reference model | 0.2.0 |
| 17/11/2023 | 126 | S4-231838 |  |  |  | Updates after adhoc calls | 0.2.0 |
| 17/11/2023 | 126 | S4-232024 |  |  |  | Updates to use cases | 0.2.0 |
| 17/11/2023 | 126 | S4-232034 |  |  |  | Updates to use cases | 0.2.0 |
| 17/11/2023 | 126 | S4-231650 |  |  |  | Updates to use cases | 0.2.0 |
| 01/02/2024 | 127 | S4-240079 |  |  |  | Updates on Reference Architecture | 0.3.0 |
| 01/02/2024 | 127 | S4-240433 |  |  |  | Avatar reference architecture details | 0.3.1 |
| 12/04/2024 | 127-e | S4aV230124 |  |  |  | Additional sets of facial landmarks | 0.4.0 |
| 23/05/2024 | 128 | S4-240928 |  |  |  | AVATAR-related features in glTF 2.0 and MPEG-I Scene Description | 0.5.0 |
| 23/05/2024 | 128 | S4-241108 |  |  |  | Morph Target Animation | 0.5.0 |
| 23/05/2024 | 128 | S4-241313 |  |  |  | Definitions and reference architecture updates | 0.5.0 |
| 23/08/2024 | 129-e | S4-241686 |  |  |  | Reference Architecture Mapping to Non-IMS Services | 0.6.0 |
| 23/08/2024 | 129-e | S4-241687 |  |  |  | MPEG Avatar Representation Format | 0.6.0 |
| 23/08/2024 | 129-e | S4-241743 |  |  |  | Mesh-based avatar protection | 0.6.0 |
| 22/11/2024 | 130 | S4-242194 |  |  |  | pCR on Avatar Representation of Avatar Properties | 0.8.0 |
| 03/12/2024 | SA#106 | SP-241755 |  |  |  | Version created by MCC to present to TSG SA for information | 1.0.0 |