| 3GPP TS 28.541 V16.6.1 (2020-11) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and SystemAspects; Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3 (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	21

Introduction	21

1	Scope	22

2	References	22

3	Definitions and abbreviations	24

3.1	Definitions	24

3.2	Abbreviations	25

4	Information model definitions for NR NRM	26

4.1	Imported and associated information	26

4.1.1	Imported information entities and local labels	26

4.1.2	Associated information entities and local labels	26

4.2	Class diagram	26

4.2.1	Class diagram for gNB and en-gNB	26

4.2.1.1	Relationships	26

4.2.1.2	Inheritance	32

4.3	Class definitions	34

4.3.1	GNBDUFunction	34

4.3.1.1	Definition	34

4.3.1.2	Attributes	35

4.3.1.3	Attribute constraints	35

4.3.1.4	Notifications	35

4.3.2	GNBCUCPFunction	35

4.3.2.1	Definition	35

4.3.2.2	Attributes	36

4.3.2.3	Attribute constraints	36

4.3.2.4	Notifications	36

4.3.3	GNBCUUPFunction	36

4.3.3.1	Definition	36

4.3.3.2	Attributes	37

4.3.3.3	Attribute constraints	37

4.3.3.4	Notifications	37

4.3.4	NRCellCU	37

4.3.4.1	Definition	37

4.3.4.2	Attributes	37

4.3.4.3	Void	38

4.3.4.4	Notifications	38

4.3.5	NRCellDU	38

4.3.5.1	Definition	38

4.3.5.2	Attributes	38

4.3.5.3	Attribute constraints	39

4.3.5.4	Notifications	39

4.3.6	NRSectorCarrier	39

4.3.6.1	Definition	39

4.3.6.2	Attributes	40

4.3.6.3	Attribute constraints	40

4.3.6.4	Notifications	40

4.3.7	BWP	40

4.3.7.1	Definition	40

4.3.7.2	Attributes	40

4.3.7.3	Attribute constraints	41

4.3.7.4	Notifications	41

4.3.8	EP_E1	41

4.3.8.1	Definition	41

4.3.8.2	Attributes	41

4.3.8.3	Attribute constraints	41

4.3.8.4	Notifications	41

4.3.9	EP_XnU	41

4.3.9.1	Definition	41

4.3.9.2	Attributes	41

4.3.9.3	Attribute constraints	42

4.3.9.4	Notifications	42

4.3.10	EP_NgC	42

4.3.10.1	Definition	42

4.3.10.2	Attributes	42

4.3.10.3	Attribute constraints	42

4.3.10.4	Notifications	42

4.3.11	EP_NgU	42

4.3.11.1	Definition	42

4.3.11.2	Attributes	42

4.3.11.3	Attribute constraints	43

4.3.11.4	Notifications	43

4.3.12	EP_F1C	43

4.3.12.1	Definition	43

4.3.12.2	Attributes	43

4.3.12.3	Attribute constraints	43

4.3.12.4	Notifications	43

4.3.13	EP_F1U	43

4.3.13.1	Definition	43

4.3.13.2	Attributes	43

4.3.13.3	Attribute constraints	44

4.3.13.4	Notifications	44

4.3.14	EP_S1U	44

4.3.14.1	Definition	44

4.3.14.2	Attributes	44

4.3.14.3	Attribute constraints	44

4.3.14.4	Notifications	44

4.3.15	EP_X2C	44

4.3.15.1	Definition	44

4.3.15.2	Attributes	44

4.3.15.3	Attribute constraints	44

4.3.15.4	Notifications	45

4.3.16	EP_X2U	45

4.3.16.1	Definition	45

4.3.16.2	Attributes	45

4.3.16.3	Attribute constraints	45

4.3.16.4	Notifications	45

4.3.17	EP_XnC	45

4.3.17.1	Definition	45

4.3.17.2	Attributes	45

4.3.17.3	Attribute constraints	45

4.3.17.4	Notifications	45

4.3.18	ExternalGNBCUCPFunction	46

4.3.18.1	Definition	46

4.3.18.2	Attributes	46

4.3.18.3	Attribute constraints	46

4.3.18.4	Notifications	46

4.3.19	ExternalGNBCUUPFunction	46

4.3.19.1	Definition	46

4.3.19.2	Attributes	46

4.3.19.3	Attribute constraints	46

4.3.19.4	Notifications	46

4.3.20	ExternalGNBDUFunction	46

4.3.20.1	Definition	46

4.3.20.2	Attributes	47

4.3.20.3	Attribute constraints	47

4.3.20.4	Notifications	47

4.3.21	ExternalUPFFunction	47

4.3.21.1	Definition	47

4.3.21.2	Attributes	47

4.3.21.3	Attribute constraints	47

4.3.21.4	Notifications	47

4.3.22	ExternalAMFFunction	47

4.3.22.1	Definition	47

4.3.22.2	Attributes	47

4.3.22.3	Attribute constraints	47

4.3.22.4	Notifications	48

4.3.23	Void	48

4.3.24	ENBFunction <<ProxyClass>>	48

4.3.24.1	Definition	48

4.3.24.2	Attributes	48

4.3.24.3	Attribute constraints	48

4.3.24.4	Notifications	48

4.3.25	GNBCUCPFunction <<ProxyClass>>	48

4.3.25.1	Definition	48

4.3.25.2	Attributes	48

4.3.25.3	Attribute constraints	48

4.3.25.4	Notifications	48

4.3.26	GNBCUUPFunction <<ProxyClass>>	48

4.3.26.1	Definition	48

4.3.26.2	Attributes	48

4.3.26.3	Attribute constraints	48

4.3.26.4	Notifications	49

4.3.27	GNBDUFunction <<ProxyClass>>	49

4.3.27.1	Definition	49

4.3.27.2	Attributes	49

4.3.27.3	Attribute constraints	49

4.3.27.4	Notifications	49

4.3.28	ServingGWFFunction <<ProxyClass>>	49

4.3.28.1	Definition	49

4.3.28.2	Attributes	49

4.3.28.3	Attribute constraints	49

4.3.28.4	Notifications	49

4.3.29	UPFFunction <<ProxyClass>>	49

4.3.29.1	Definition	49

4.3.29.2	Attributes	49

4.3.29.3	Attribute constraints	49

4.3.29.4	Notifications	50

4.3.30	AMFFunction <<ProxyClass>>	50

4.3.30.1	Definition	50

4.3.30.2	Attributes	50

4.3.30.3	Attribute constraints	50

4.3.30.4	Notifications	50

4.3.31	Void	50

4.3.32	NRCellRelation	50

4.3.32.1	Definition	50

4.3.32.2	Attributes	51

4.3.32.3	Attribute constraints	51

4.3.32.4	Notifications	51

4.3.33	NRFreqRelation	51

4.3.33.1	Definition	51

4.3.33.2	Attributes	51

4.3.33.3	Attribute constraints	51

4.3.33.4	Notifications	52

4.3.34	Void	52

4.3.35	ExternalNRCellCU	52

4.3.35.1	Definition	52

4.3.35.2	Attributes	52

4.3.35.3	Attribute constraints	52

4.3.35.4	Notifications	52

4.3.36	RRMPolicyRatio	52

4.3.36.1	Definition	52

4.3.36.2	Attributes	53

4.3.36.3	Attribute constraints	53

4.3.36.4	Notifications	53

4.3.37	S-NSSAI <<dataType>>	53

4.3.37.1	Definition	53

4.3.37.2	Attributes	53

4.3.37.3	Attribute constraints	54

4.3.37.4	Notifications	54

4.3.38	NRFrequency	54

4.3.38.1	Definition	54

4.3.38.2	Attributes	54

4.3.38.3	Attribute constraints	54

4.3.38.4	Notifications	54

4.3.39	CommonBeamformingFunction	54

4.3.39.1	Definition	54

4.3.39.2	Attributes	55

4.3.39.3	Attribute constraints	55

4.3.39.4	Notifications	55

4.3.40	Beam	55

4.3.40.1	Definition	55

4.3.40.2	Attributes	55

4.3.40.3	Attribute constraints	55

4.3.41	PLMNInfo <<dataType>>	56

4.3.41.1	Definition	56

4.3.41.2	Attributes	56

4.3.41.3	Attribute constraints	56

4.3.41.4	Notifications	56

4.3.42	RRMPolicyMember <<dataType>>	56

4.3.42.1	Definition	56

4.3.42.2	Attributes	56

4.3.42.3	Attribute constraints	56

4.3.42.4	Notifications	56

4.3.43	RRMPolicy_	57

4.3.43.1	Definition	57

4.3.43.2	Attributes	57

4.3.43.3	Attribute constraints	57

4.3.43.4	Notifications	57

4.3.44	RRMPolicyManagedEntity <<ProxyClass>>	57

4.3.44.1	Definition	57

4.3.44.2	Attributes	58

4.3.44.3	Attribute constraints	58

4.3.44.4	Notifications	58

4.3.45	GNBCUCPNeighbour <<ProxyClass>>	58

4.3.45.1	Definition	58

4.3.45.2	Attributes	58

4.3.45.3	Attribute constraints	58

4.3.45.4	Notifications	58

4.3.46	GNBCUUPNeighbour <<ProxyClass>>	58

4.3.46.1	Definition	58

4.3.46.2	Attributes	58

4.3.46.3	Attribute constraints	58

4.3.46.4	Notifications	58

4.3.47	MappingSetIDBackhaulAddress  <<dataType>>	59

4.3.47.1	Definition	59

4.3.47.2	Attributes	59

4.3.47.3	Attribute constraints	59

4.3.47.4	Notifications	59

4.3.48	BackhaulAddress  <<dataType>>	59

4.3.48.1	Definition	59

4.3.48.2	Attributes	59

4.3.48.3	Attribute constraints	59

4.3.48.4	Notifications	59

4.3.49	TAI  <<dataType>>	59

4.3.49.1	Definition	59

4.3.49.2	Attributes	59

4.3.49.3	Attribute constraints	59

4.3.49.4	Notifications	60

4.3.51	FrequencyDomainPara <<dataType>>	61

4.3.51.1	Definition	61

4.3.51.2	Attributes	61

4.3.51.3	Attribute constraints	61

4.3.51.4	Notifications	61

4.3.52	SequenceDomainPara  <<dataType>>	61

4.3.52.1	Definition	61

4.3.52.2	Attributes	61

4.3.52.3	Attribute constraints	61

4.3.52.4	Notifications	61

4.3.53	TimeDomainPara  <<dataType>>	61

4.3.53.1	Definition	61

4.3.53.2	Attributes	61

4.3.53.3	Attribute constraints	62

4.3.53.4	Notifications	62

4.3.54	RimRSReportConf <<dataType>>	62

4.3.54.1	Definition	62

4.3.54.2	Attributes	62

4.3.54.3	Attribute constraints	62

4.3.54.4	Notifications	62

4.3.55	RimRSReportInfo <<dataType>>	62

4.3.55.1	Definition	62

4.3.55.2	Attributes	63

4.3.55.3	Attribute constraints	63

4.3.55.4	Notifications	63

4.3.57	DANRManagementFunction	63

4.3.57.1	Definition	63

4.3.57.2	Attributes	64

4.3.57.3	Attribute constraints	64

4.3.57.4	Notifications	64

4.3.58	DESManagementFunction	64

4.3.58.1	Definition	64

4.3.58.2	Attributes	64

4.3.58.3	Attribute constraints	65

4.3.58.4	Notification	65

4.3.59	DRACHOptimizationFunction	65

4.3.59.1	Definition	65

4.3.59.2	Attributes	65

4.3.59.3	Attribute constraints	65

4.3.59.4	Notifications	65

4.3.60	DMROFunction	66

4.3.60.1	Definition	66

4.3.60.2	Attributes	66

4.3.60.3	Attribute constraints	66

4.3.60.4	Notifications	66

4.3.61	DPCIConfigurationFunction	66

4.3.61.1	Definition	66

4.3.61.2	Attributes	66

4.3.61.3	Attribute constraints	66

4.3.61.4	Notifications	66

4.3.62	CPCIConfigurationFunction	67

4.3.62.1	Definition	67

4.3.62.2	Attributes	67

4.3.62.3	Attribute constraints	67

4.3.62.4	Notifications	67

4.3.63	CESManagementFunction	67

4.3.63.1	Definition	67

4.3.63.2	Attributes	67

4.3.63.3	Attribute constraints	67

4.3.63.4	Notification	68

4.3.64	AddressWithVlan <<dataType>>	68

4.3.64.1	Definition	68

4.3.64.2	Attributes	68

4.3.64.3	Attribute constraints	68

4.3.64.4	Notifications	68

4.3.65	TceIDMappingInfo  <<dataType>>	68

4.3.65.1	Definition	68

4.3.65.2	Attributes	68

4.3.65.3	Attribute constraints	68

4.3.65.4	Notifications	68

4.4	Attribute definitions	69

4.4.1	Attribute properties	69

4.5	Common notifications	98

4.5.1	Alarm notifications	98

4.5.2	Configuration notifications	98

5	Information Model definitions for 5GC NRM	99

5.1	Imported information entities and local labels	99

5.2	Class diagram	99

5.2.1	Class diagram of 5GC NFs	99

5.2.1.1	Relationships	99

5.2.1.2	Inheritance	106

5.2.2	Class diagram of AMF Region/AMF Set	109

5.2.2.1	Relationships	109

5.2.2.2	Inheritance	110

5.3	Class definitions	110

5.3.1	AMFFunction	110

5.3.1.1	Definition	110

5.3.1.2	Attributes	110

5.3.1.3	Attribute constraints	111

5.3.1.4	Notifications	111

5.3.2	SMFFunction	111

5.3.2.1	Definition	111

5.3.2.2	Attributes	111

5.3.2.3	Attribute constraints	111

5.3.2.4	Notifications	111

5.3.3	UPFFunction	111

5.3.3.1	Definition	111

5.3.3.2	Attributes	111

5.3.3.3	Attribute constraints	112

5.3.3.4	Notifications	112

5.3.4	N3IWFFunction	112

5.3.4.1	Definition	112

5.3.4.2	Attributes	112

5.3.4.3	Attribute constraints	112

5.3.4.4	Notifications	112

5.3.5	PCFFunction	112

5.3.5.1	Definition	112

5.3.5.2	Attributes	112

5.3.5.3	Attribute constraints	113

5.3.5.4	Notifications	113

5.3.6	AUSFFunction	113

5.3.6.1	Definition	113

5.3.6.2	Attributes	113

5.3.6.3	Attribute constraints	113

5.3.6.4	Notifications	113

5.3.7	UDMFunction	113

5.3.7.1	Definition	113

5.3.7.2	Attributes	113

5.3.5.3	Attribute constraints	114

5.3.5.4	Notifications	114

5.3.8	UDRFunction	114

5.3.8.1	Definition	114

5.3.8.2	Attributes	114

5.3.8.3	Attribute constraints	114

5.3.8.4	Notifications	114

5.3.9	UDSFFunction	114

5.3.9.1	Definition	114

5.3.9.2	Attributes	114

5.3.9.3	Attribute constraints	115

5.3.9.4	Notifications	115

5.3.10	NRFFunction	115

5.3.10.1	Definition	115

5.3.10.2	Attributes	115

5.3.10.3	Attribute constraints	115

5.3.10.4	Notifications	115

5.3.11	NSSFFunction	115

5.3.11.1	Definition	115

5.3.11.2	Attributes	115

5.3.11.3	Attribute constraints	116

5.3.11.4	Notifications	116

5.3.12	AFFunction	116

5.3.12.1	Definition	116

5.3.13	DNFunction	116

5.3.13.1	Definition	116

5.3.14	SMSFFunction	116

5.3.14.1	Definition	116

5.3.14.2	Attributes	116

5.3.14.3	Attribute constraints	116

5.3.14.4	Notifications	116

5.3.15	LMFFunction	117

5.3.15.1	Definition	117

5.3.15.2	Attributes	117

5.3.15.3	Attribute constraints	117

5.3.15.4	Notifications	117

5.3.16	NGEIRFunction	117

5.3.16.1	Definition	117

5.3.16.2	Attributes	117

5.3.16.3	Attribute constraints	117

5.3.16.4	Notifications	117

5.3.17	SEPPFunction	117

5.3.17.1	Definition	117

5.3.17.2	Attributes	118

5.3.17.3	Attribute constraints	118

5.3.17.4	Notifications	118

5.3.18	NWDAFFunction	118

5.3.18.1	Definition	118

5.3.18.2	Attributes	118

5.3.18.3	Attribute constraints	118

5.3.18.4	Notifications	118

5.3.19	EP_N2	118

5.3.19.1	Definition	118

5.3.19.2	Attributes	118

5.3.19.3	Attribute constraints	119

5.3.19.4	Notifications	119

5.3.20	EP_N3	119

5.3.20.1	Definition	119

5.3.20.2	Attributes	119

5.3.20.3	Attribute constraints	119

5.3.20.4	Notifications	119

5.3.21	EP_N4	119

5.3.21.1	Definition	119

5.3.21.2	Attributes	119

5.3.21.3	Attribute constraints	119

5.3.21.4	Notifications	119

5.3.22	EP_N5	120

5.3.22.1	Definition	120

5.3.22.2	Attributes	120

5.3.22.3	Attribute constraints	120

5.3.22.4	Notifications	120

5.3.23	EP_N6	120

5.3.23.1	Definition	120

5.3.23.2	Attributes	120

5.3.23.3	Attribute constraints	120

5.3.23.4	Notifications	120

5.3.24	EP_N7	120

5.3.24.1	Definition	120

5.3.24.2	Attributes	120

5.3.24.3	Attribute constraints	121

5.3.24.4	Notifications	121

5.3.25	EP_N8	121

5.3.25.1	Definition	121

5.3.25.2	Attributes	121

5.3.25.3	Attribute constraints	121

5.3.25.4	Notifications	121

5.3.26	EP_N9	121

5.3.26.1	Definition	121

5.3.26.2	Attributes	121

5.3.26.3	Attribute constraints	121

5.3.26.4	Notifications	121

5.3.27	EP_N10	122

5.3.27.1	Definition	122

5.3.27.2	Attributes	122

5.3.27.3	Attribute constraints	122

5.3.27.4	Notifications	122

5.3.28	EP_N11	122

5.3.28.1	Definition	122

5.3.28.2	Attributes	122

5.3.28.3	Attribute constraints	122

5.3.28.4	Notifications	122

5.3.29	EP_N12	122

5.3.29.1	Definition	122

5.3.29.2	Attributes	122

5.3.29.3	Attribute constraints	123

5.3.29.4	Notifications	123

5.3.30	EP_N13	123

5.3.30.1	Definition	123

5.3.30.2	Attributes	123

5.3.30.3	Attribute constraints	123

5.3.30.4	Notifications	123

5.3.31	EP_N14	123

5.3.31.1	Definition	123

5.3.31.2	Attributes	123

5.3.31.3	Attribute constraints	123

5.3.31.4	Notifications	123

5.3.32	EP_N15	124

5.3.32.1	Definition	124

5.3.32.2	Attributes	124

5.3.32.3	Attribute constraints	124

5.3.32.4	Notifications	124

5.3.33	EP_N16	124

5.3.33.1	Definition	124

5.3.33.2	Attributes	124

5.3.33.3	Attribute constraints	124

5.3.33.4	Notifications	124

5.3.34	EP_N17	124

5.3.34.1	Definition	124

5.3.34.2	Attributes	124

5.3.34.3	Attribute constraints	125

5.3.34.4	Notifications	125

5.3.35	EP_N20	125

5.3.35.1	Definition	125

5.3.35.2	Attributes	125

5.3.35.3	Attribute constraints	125

5.3.35.4	Notifications	125

5.3.36	EP_N21	125

5.3.36.1	Definition	125

5.3.36.2	Attributes	125

5.3.36.3	Attribute constraints	125

5.3.36.4	Notifications	125

5.3.37	EP_N22	126

5.3.37.1	Definition	126

5.3.37.2	Attributes	126

5.3.37.3	Attribute constraints	126

5.3.37.4	Notifications	126

5.3.38	EP_N26	126

5.3.38.1	Definition	126

5.3.38.2	Attributes	126

5.3.38.3	Attribute constraints	126

5.3.38.4	Notifications	126

5.3.39	Void	126

5.3.40	Void	126

5.3.41	EP_S5C	126

5.3.41.1	Definition	126

5.3.41.2	Attributes	127

5.3.41.3	Attribute constraints	127

5.3.41.4	Notifications	127

5.3.42	EP_S5U	127

5.3.42.1	Definition	127

5.3.42.2	Attributes	127

5.3.42.3	Attribute constraints	127

5.3.42.4	Notifications	127

5.3.43	EP_Rx	127

5.3.43.1	Definition	127

5.3.43.2	Attributes	127

5.3.43.3	Attribute constraints	127

5.3.43.4	Notifications	128

5.3.44	EP_MAP_SMSC	128

5.3.44.1	Definition	128

5.3.44.2	Attributes	128

5.3.44.3	Attribute constraints	128

5.3.44.4	Notifications	128

5.3.45	EP_NLS	128

5.3.45.1	Definition	128

5.3.45.2	Attributes	128

5.3.45.3	Attribute constraints	128

5.3.45.4	Notifications	128

5.3.46	EP_NLG	128

5.3.46.1	Definition	128

5.3.46.2	Attributes	129

5.3.46.3	Attribute constraints	129

5.3.46.4	Notifications	129

5.3.47	EP_N27	129

5.3.47.1	Definition	129

5.3.47.2	Attributes	129

5.3.47.3	Attribute constraints	129

5.3.47.4	Notifications	129

5.3.48	EP_N31	129

5.3.48.1	Definition	129

5.3.48.2	Attributes	129

5.3.48.3	Attribute constraints	129

5.3.48.4	Notifications	130

5.3.49	ExternalNRFFunction	130

5.3.49.1	Definition	130

5.3.49.2	Attributes	130

5.3.49.3	Attribute constraints	130

5.3.49.4	Notifications	130

5.3.50	ExternalNSSFFunction	130

5.3.50.1	Definition	130

5.3.50.2	Attributes	130

5.3.50.3	Attribute constraints	130

5.3.50.4	Notifications	130

5.3.51	AMFSet	131

5.3.51.1	Definition	131

5.3.51.2	Attributes	131

5.3.51.3	Attribute constraints	131

5.3.51.4	Notifications	131

5.3.52	AMFRegion	131

5.3.52.1	Definition	131

5.3.52.2	Attributes	131

5.3.52.3	Attribute constraints	131

5.3.52.4	Notifications	131

5.3.53	ExternalAMFFunction	132

5.3.53.1	Definition	132

5.3.53.2	Attributes	132

5.3.53.3	Attribute constraints	132

5.3.53.4	Notifications	132

5.3.54	ManagedNFProfile <<dataType>>	132

5.3.54.1	Definition	132

5.3.54.2	Attributes	132

5.3.54.3	Attribute constraints	132

5.3.54.4	Notifications	132

5.3.55	HostAddr <<choice>>	132

5.3.55.1	Definition	132

5.3.56	NFInfo <<choice>>	133

5.3.56.1	Definition	133

5.3.57	UdmInfo <<dataType>>	133

5.3.57.1	Definition	133

5.3.57.2	Attributes	133

5.3.57.3	Attribute constraints	134

5.3.57.4	Notifications	134

5.3.58	AusfInfo <<dataType>>	134

5.3.58.1	Definition	134

5.3.58.2	Attributes	134

5.3.58.3	Attribute constraints	134

5.3.58.4	Notifications	134

5.3.59	UpfInfo <<dataType>>	134

5.3.59.1	Definition	134

5.3.59.2	Attributes	134

5.3.59.3	Attribute constraints	134

5.3.59.4	Notifications	134

5.3.60	AmfInfo <<dataType>>	134

5.3.60.1	Definition	134

5.3.60.2	Attributes	135

5.3.60.3	Attribute constraints	135

5.3.60.4	Notifications	135

5.3.61	Udrinfo <<dataType>>	135

5.3.61.1	Definition	135

5.3.61.2	Attributes	135

5.3.61.3	Attribute constraints	135

5.3.61.4	Notifications	135

5.3.62	EP_N32	135

5.3.62.1	Definition	135

5.3.62.2	Attributes	135

5.3.62.3	Attribute constraints	135

5.3.62.4	Notifications	136

5.3.63	ExternalSEPPFunction	136

5.3.63.1	Definition	136

5.3.63.2	Attributes	136

5.3.63.3	Attribute constraints	136

5.3.63.4	Notifications	136

5.3.64	SEPPFunction <<ProxyClass>>	136

5.3.64.1	Definition	136

5.3.64.2	Attributes	136

5.3.64.3	Attribute constraints	136

5.3.64.4	Notifications	136

5.3.65	NEFFunction	136

5.3.65.1	Definition	136

5.3.65.2	Attributes	136

5.3.65.3	Attribute constraints	137

5.3.65.4	Notifications	137

5.3.66	SCPFunction	137

5.3.67.1	Definition	137

5.3.67.2	Attributes	137

5.3.67.3	Attribute constraints	137

5.3.67.4	Notifications	137

5.3.68	SupportedFunction <<dataType>>	137

5.3.68.1	Definition	137

5.3.68.2	Attributes	137

5.3.68.3	Attribute constraints	137

5.3.68.4	Notifications	137

5.3.69	CommModel <<dataType>>	138

5.3.69.1	Definition	138

5.3.69.2	Attributes	138

5.3.69.3	Attribute constraints	138

5.3.69.4	Notifications	138

5.3.70	QFQoSMonitoringControl	138

5.3.70.1	Definition	138

5.3.70.2	Attributes	138

5.3.70.3	Attribute constraints	138

5.3.70.4	Notifications	138

5.3.71	QFDelayThresholdsType <<dataType>>	139

5.3.71.1	Definition	139

5.3.71.2	Attributes	139

5.3.71.3	Attribute constraints	139

5.3.71.4	Notifications	139

5.3.72	GtpUPathQoSMonitoringControl	139

5.3.72.1	Definition	139

5.3.72.2	Attributes	140

5.3.72.3	Attribute constraints	140

5.3.72.4	Notifications	140

5.3.73	GtpUPathDelayThresholdsType <<dataType>>	140

5.3.73.1	Definition	140

5.3.73.2	Attributes	140

5.3.73.3	Attribute constraints	140

5.3.73.4	Notifications	140

5.3.75	Configurable5QISet	141

5.3.75.1	Definition	141

5.3.75.2	Attributes	141

5.3.75.3	Attribute constraints	141

5.3.75.4	Notifications	141

5.3.76	FiveQICharacteristics <<dataType>>	141

5.3.76.1	Definition	141

5.3.76.2	Attributes	141

5.3.76.3	Attribute constraints	141

5.3.76.4	Notifications	141

5.3.77	PacketErrorRate <<dataType>>	141

5.3.77.1	Definition	141

5.3.77.2	Attributes	142

5.3.77.3	Attribute constraints	142

5.3.77.4	Notifications	142

5.3.78	FiveQiDscpMappingSet	142

5.3.78.1	Definition	142

5.3.78.2	Attributes	142

5.3.78.3	Attribute constraints	142

5.3.78.4	Notifications	142

5.3.79	FiveQiDscpMapping <<dataType>>	142

5.3.79.1	Definition	142

5.3.79.2	Attributes	142

5.3.79.3	Attribute constraints	142

5.3.79.4	Notifications	142

5.3.80	PredefinedPccRuleSet	143

5.3.80.1	Definition	143

5.3.80.2	Attributes	143

5.3.80.3	Attribute constraints	143

5.3.80.4	Notifications	143

5.3.81	PccRule <<dataType>>	143

5.3.81.1	Definition	143

5.3.81.2	Attributes	143

5.3.81.3	Attribute constraints	143

5.3.81.4	Notifications	144

5.3.82	FlowInformation <<dataType>>	144

5.3.82.1	Definition	144

5.3.82.2	Attributes	144

5.3.82.3	Attribute constraints	144

5.3.82.4	Notifications	144

5.3.83	EthFlowDescription <<dataType>>	144

5.3.83.1	Definition	144

5.3.83.2	Attributes	144

5.3.83.3	Attribute constraints	145

5.3.83.4	Notifications	145

5.3.84	QoSData <<dataType>>	145

5.3.84.1	Definition	145

5.3.84.2	Attributes	145

5.3.84.3	Attribute constraints	145

5.3.84.4	Notifications	145

5.3.85	ARP <<dataType>>	145

5.3.85.1	Definition	145

5.3.85.2	Attributes	145

5.3.85.3	Attribute constraints	146

5.3.85.4	Notifications	146

5.3.86	TrafficControlData <<dataType>>	146

5.3.86.1	Definition	146

5.3.86.2	Attributes	146

5.3.86.3	Attribute constraints	146

5.3.86.4	Notifications	146

5.3.87	RedirectInformation <<dataType>>	146

5.3.87.1	Definition	146

5.3.87.2	Attributes	147

5.3.87.3	Attribute constraints	147

5.3.87.4	Notifications	147

5.3.88	RouteToLocation <<dataType>>	147

5.3.88.1	Definition	147

5.3.88.2	Attributes	147

5.3.88.3	Attribute constraints	147

5.3.88.4	Notifications	147

5.3.89	RouteInformation <<dataType>>	147

5.3.89.1	Definition	147

5.3.89.2	Attributes	147

5.3.89.3	Attribute constraints	148

5.3.89.4	Notifications	148

5.3.90	UpPathChgEvent <<dataType>>	148

5.3.90.1	Definition	148

5.3.90.2	Attributes	148

5.3.90.3	Attribute constraints	148

5.3.90.4	Notifications	148

5.3.91	SteeringMode <<dataType>>	148

5.3.91.1	Definition	148

5.3.91.2	Attributes	148

5.3.91.3	Attribute constraints	149

5.3.91.4	Notifications	149

5.3.92	ConditionData  <<dataType>>	149

5.3.92.1	Definition	149

5.3.92.2	Attributes	149

5.3.92.3	Attribute constraints	149

5.3.92.4	Notifications	149

5.3.93	TscaiInputContainer  <<dataType>>	149

5.3.93.1	Definition	149

5.3.93.2	Attributes	150

5.3.93.3	Attribute constraints	150

5.3.93.4	Notifications	150

5.3.94	Dynamic5QISet	150

5.3.94.1	Definition	150

5.3.94.2	Attributes	150

5.3.94.3	Attribute constraints	150

5.3.94.4	Notifications	150

5.4	Attribute definitions	150

5.4.1	Attribute properties	150

5.5	Common notifications	169

5.5.1	Alarm notifications	169

5.5.2	Configuration notifications	170

6	Information model definitions for network slice NRM	170

6.1	Imported information entities and local labels	170

6.2	Class diagram	171

6.2.1	Relationships	171

6.2.2	Inheritance	172

6.3	Class definitions	172

6.3.1	NetworkSlice	172

6.3.1.1	Definition	172

6.3.1.2	Attributes	172

6.3.1.3	Attribute constraints	172

6.3.1.4	Notifications	172

6.3.2	NetworkSliceSubnet	173

6.3.2.1	Definition	173

6.3.2.2	Attributes	173

6.3.2.3	Attribute constraints	173

6.3.2.4	Notifications	173

6.3.3	ServiceProfile <<dataType>>	173

6.3.3.1	Definition	173

6.3.3.2	Attributes	174

6.3.3.3	Attribute constraints	174

6.3.3.4	Notifications	174

6.3.4	SliceProfile <<dataType>>	174

6.3.4.1	Definition	174

6.3.4.2	Attributes	175

6.3.4.3	Attribute constraints	175

6.3.4.4	Notifications	175

6.3.5	NsInfo <<dataType>>	175

6.3.5.1	Definition	175

6.3.5.2	Attributes	175

6.3.5.3	Attribute constraints	175

6.3.5.4	Notifications	175

6.3.6	ServAttrCom <<dataType>>	175

6.3.6.1	Definition	175

6.3.6.2	Attributes	176

6.3.6.3	Attribute constraints	176

6.3.6.4	Notifications	176

6.3.7	DelayTolerance<<dataType>>	176

6.3.7.1	Definition	176

6.3.7.2	Attributes	176

6.3.7.3	Attribute constraints	176

6.3.7.4	Notifications	176

6.3.7	DeterminComm <<dataType>>	176

6.3.7.1	Definition	176

6.3.7.2	Attributes	176

6.3.7.3	Attribute constraints	176

6.3.7.4	Notifications	177

6.3.8	DLThpt<<dataType>>	177

6.3.8.1	Definition	177

6.3.8.2	Attributes	177

6.3.8.3	Attribute constraints	177

6.3.8.4	Notifications	177

6.3.9	ULThpt<<dataType>>	177

6.3.9.1	Definition	177

6.3.9.2	Attributes	177

6.3.9.3	Attribute constraints	177

6.3.9.4	Notifications	177

6.3.10	MaxPktSize <<dataType>>	177

6.3.10.1	Definition	177

6.3.10.2	Attributes	178

6.3.10.3	Attribute constraints	178

6.3.10.4	Notifications	178

6.3.11	MaxNumberofConns <<dataType>>	178

6.3.11.1	Definition	178

6.3.11.2	Attributes	178

6.3.11.3	Attribute constraints	178

6.3.11.4	Notifications	178

6.3.12	Void	178

6.3.13	KPIMonitoring <<dataType>>	178

6.3.13.1	Definition	178

6.3.13.2	Attributes	178

6.3.13.3	Attribute constraints	178

6.3.13.4	Notifications	178

6.3.14	UserMgmtOpen<<dataType>>	179

6.3.14.1	Definition	179

6.3.14.2	Attributes	179

6.3.14.3	Attribute constraints	179

6.3.14.4	Notifications	179

6.3.15	V2XCommMode<<dataType>>	179

6.3.15.1	Definition	179

6.3.15.2	Attributes	179

6.3.15.3	Attribute constraints	179

6.3.15.4	Notifications	179

6.3.16	TermDensity<<dataType>>	179

6.3.16.1	Definition	179

6.3.16.2	Attributes	179

6.3.16.3	Attribute constraints	179

6.3.16.4	Notifications	180

6.3.17	EP_Transport	180

6.3.17.1	Definition	180

6.3.17.2	Attributes	180

6.3.17.3	Attribute constraints	180

6.3.17.4	Notifications	180

6.3.18	EP_Application <<ProxyClass>>	180

6.3.18.1	Definition	180

6.3.18.2	Attributes	180

6.3.18.3	Attribute constraints	180

6.3.18.4	Notifications	180

6.4	Attribute definition	181

6.4.1	Attribute properties	181

6.5	Common notifications	188

6.5.1	Alarm notifications	188

6.5.2	Configuration notifications	189

7	Solution Set (SS)	189

Annex A (normative): Cell state handling	190

A.1	Relation between the administrative state and the "Pre-operation state of the gNB-DU Cell"	190

A.2	Combined state diagram for gNB cell	190

Annex B (normative):  NSI and NSSI state handling	194

B.1	NSI state handling	194

B.2	State handling of NSSI	195

Annex C (normative): XML definitions for NR NRM	198

C.1	General	198

C.2	Architectural features	198

C.3	Mapping	198

C.3.1	General mapping	198

C.3.2	Information Object Class (IOC) mapping	198

C.4	Solution Set definitions	198

C.4.1	XML definition structure	198

C.4.2	Graphical representation	198

C.4.3	XML schema "nRNrm.xsd"	199

Annex D (normative): OpenAPI definition of the NR NRM	221

D.1	General	221

D.2	Void	221

D.3	Void	221

D.4	Solution Set (SS) definitions	221

D.4.1	Void	221

D.4.2	Void	221

D.4.3	OpenAPI document "nrNrm.yaml"	221

Annex E (normative): YANG definitions for NR NRM	246

E.1	General	246

E.2	Void	246

E.3	Void	246

E.4	Void	246

E.5	Modules	246

E.5.1	module _3gpp-nr-nrm-beam@2019-11-22.yang	246

E.5.1a	module _3gpp-nr-nrm-bwp@2019-10-28.yang	248

E.5.1b	module _3gpp-nr-nrm-commonbeamformingfunction@2019-11-22.yang	249

E.5.2	module_3gpp-nr-nrm-ep@.yang	250

E.5.3	module _3gpp-nr-nrm-eutrancellrelation@2019-10-28.yang	253

E.5.4	module _3gpp-nr-nrm-eutranetwork@2019-06-17.yang	255

E.5.5	module _3gpp-nr-nrm-eutranfreqrelation@2019-10-28.yang	256

E.5.6	module _3gpp-nr-nrm-eutranfrequency@2019-10-28.yang	259

E.5.7	module _3gpp-nr-nrm-externalamffunction@2019-10-28.yang	260

E.5.8	module _3gpp-nr-nrm-externalenbfunction@2019-10-28.yang	261

E.5.9	module_3gpp-nr-nrm-externaleutrancell@2019-10-28.yang	261

E.5.10	module _3gpp-nr-nrm-externalgnbcucpfunction@2019-10-28.yang	264

E.5.11	module _3gpp-nr-nrm-externalgnbcuupfunction@2019-10-28.yang	265

E.5.12	module _3gpp-nr-nrm-externalgnbdufunction@2019-10-28.yang	266

E.5.13	module _3gpp-nr-nrm-externalnrcellcu@2019-10-28.yang	267

E.5.14	module _3gpp-nr-nrm-externalservinggwfunction@2019-10-28.yang	268

E.5.15	module _3gpp-nr-nrm-externalupffunction@2019-10-28.yang	268

E.5.16	module _3gpp-nr-nrm-gnbcucpfunction.yang	269

E.5.17	module _3gpp-nr-nrm-gnbcuupfunction.yang	271

E.5.18	module_3gpp-nr-nrm-gnbdufunction@2020-02-14.yang	273

E.5.19	module _3gpp-nr-nrm-nrcellcu@2020-02-14.yang	274

E.5.20	module _3gpp-nr-nrm-nrcelldu@2020-02-14.yang	276

E.5.21	module _3gpp-nr-nrm-nrcellrelation@2019-10-28.yang	279

E.5.22	module _3gpp-nr-nrm-nrfreqrelation@2019-10-28.yang	280

E.5.23	module _3gpp-nr-nrm-nrfrequency@2019-10-28.yang	284

E.5.24	module _3gpp-nr-nrm-nrnetwork@2019-06-17.yang	285

E.5.25	module _3gpp-nr-nrm-nrsectorcarrier.yang	285

E.5.26	module _3gpp-nr-nrm-rrmpolicy.yang	287

E.5.27	module _3gpp-nr-nrm-common@2020-02-14.yang	288

E.5.28	module _3gpp-nr-nrm-danrmanagementfunction.yang	289

E.5.29	module _3gpp-nr-nrm-desmanagementfunction.yang	289

E.5.30	module _3gpp-nr-nrm-drachoptimizationfunction.yang	293

E.5.31	module _3gpp-nr-nrm-dmrofunction.yang	295

E.5.32	module _3gpp-nr-nrm-dpciconfigurationfunction.yang	296

E.5.33	module _3gpp-nr-nrm-cpciconfigurationfunction.yang	297

E.5.34	module _3gpp-nr-nrm-cesmanagementfunction.yang	298

E.6	Void	299

E.7	Mount information	299

Annex F (normative): XML definitions for 5GC NRM	300

F.1	General	300

F.2	Architectural features	300

F.3	Mapping	300

F.3.1	General mapping	300

F.3.2	Information Object Class (IOC) mapping	300

F.4	Solution Set definitions	300

F.4.1	XML definition structure	300

F.4.2	Graphical representation	300

F.4.3	XML schema "ngcNrm.xsd"	301

Annex G (normative): OpenAPI definition of the 5GC NRM	331

G.1	General	331

G.2	Void	331

G.3	Void	331

G.4	Solution Set (SS) definitions	331

G.4.1	Void	331

G.4.2	Void	331

G.4.3	OpenAPI document "5gcNrm.yaml"	331

Annex H (normative): YANG definitions for 5GC	357

H.1	General	357

H.2	Void	357

H.3	Void	357

H.4	Void	357

H.5	Modules	357

H.5.1	module _3gpp-5gc-common-yang-types@2019-10-20.yang	357

H.5.1a	module _3gpp-5gc-nrm-affunction@2019-10-28.yang	358

H.5.2	module _3gpp-5gc-nrm-amffunction@2019-10-25.yang	359

H.5.3	module _3gpp-5gc-nrm-amfregion@2019-10-28.yang	360

H.5.4	module _3gpp-5gc-nrm-amfset@2019-10-28.yang	361

H.5.5	module _3gpp-5gc-nrm-ausffunction@2019-10-25.yang	362

H.5.6	module _3gpp-5gc-nrm-dnfunction@2019-10-28.yang	363

H.5.7	module _3gpp-5gc-nrm-ep@2019-11-18.yang	364

H.5.8	module _3gpp-5gc-nrm-externalnrffunction@2019-10-28.yang	372

H.5.9	module _3gpp-5gc-nrm-externalnssffunction@2019-10-28.yang	373

H.5.10	module _3gpp-5gc-nrm-lmffunction@2019-10-25.yang	373

H.5.11	module _3gpp-5gc-nrm-n3iwffunction@2019-10-28.yang	374

H.5.12	module _3gpp-5gc-nrm-nfprofile@2019-06-17.yang	375

H.5.13	module _3gpp-5gc-nrm-nfservice@2019-06-17.yang	390

H.5.14	module _3gpp-5gc-nrm-ngeirfunction@2019-10-25.yang	394

H.5.15	module _3gpp-5gc-nrm-nrffunction.yang	395

H.5.16	module _3gpp-5gc-nrm-nssffunction.yang	396

H.5.17	module _3gpp-5gc-nrm-nwdaffunction@2019-10-25.yang	397

H.5.18	module _3gpp-5gc-nrm-pcffunction@2020-08-06.yang	398

H.5.19	module _3gpp-5gc-nrm-seppfunction.yang	399

H.5.19a	module _3gpp-5gc-nrm- externalseppfunction@2019-11-17.yang	400

H.5.20	module _3gpp-5gc-nrm-smffunction@2020-08-06.yang	401

H.5.21	module _3gpp-5gc-nrm-smsffunction@2019-10-25.yang	402

H.5.22	module _3gpp-5gc-nrm-udmfunction@2019-10-25.yang	403

H.5.23	module _3gpp-5gc-nrm-udrfunction@2019-10-25.yang	404

H.5.24	module _3gpp-5gc-nrm-udsffunction@2019-10-25.yang	405

H.5.25	module _3gpp-5gc-nrm-upffunction@2019-10-25.yang	406

H.5.26	module _3gpp-5gc-nrm-scpfunction.yang	407

H.5.27	module _3gpp-5gc-nrm-neffunction.yang	408

H.5.28	module _3gpp-5gc-nrm-QFQoSMonitoringControl.yang	409

H.5.29	module _3gpp-5gc-nrm-GtpUPathQoSMonitoringControl.yang	411

H.5.30	module _3gpp-5gc-nrm-Configurable5QISet.yang	413

H.5.31	module _3gpp-5gc-nrm-FiveQiDscpMappingSet.yang	414

H.5.32	module _3gpp-5gc-nrm-PredefinedPccRuleSet.yang	415

H.5.33	module _3gpp-5gc-nrm-dynamic5QISet@2020-08-06.yang	424

H.6	Void	425

H.7	Mount information	425

Annex I (normative): XML definitions for network slice	426

I.1	General	426

I.2	Architectural features	426

I.3	Mapping	426

I.3.1	General mapping	426

I.3.2	Information Object Class (IOC) mapping	426

I.4	Solution Set (SS) definitions	426

I.4.1	XML definition structure	426

I.4.2	Graphical representation	426

I.4.3	XML schema "sliceNrm.xsd"	427

Annex J (normative): OpenAPI definition of the Slice NRM	431

J.1	General	431

J.2	Void	431

J.3	Void	431

J.4	Solution Set (SS) definitions	431

J.4.1	Void	431

J.4.2	Void	431

J.4.3	OpenAPI document "sliceNrm.yaml"	431

Annex K (normative): Void	437

Annex L (normative):  Relation of GSMA GST, ServiceProfile and SliceProfile	438

L.1	General	438

L.2	GSMA GST, ServiceProfile and sliceProfile	438

Annex M (normative):  Managed NF Service state handling	439

M.1	Combined state diagram for a Managed NF Service	439

Annex N (informative): Change history	441

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document is part of a TS-family covering the 3rd Generation Partnership Project Technical Specification Group Services and System Aspects Management and orchestration of networks, as identified below:

TS 28.540:	Management and orchestration of 5G networks; Network Resource Model (NRM); Stage 1.

TS 28.541:	Management and orchestration of 5G networks; Network Resource Model (NRM); Stage 2 and stage 3.

# 1	Scope

The present document specifies the Information Model and Solution Set for the Network Resource Model (NRM) definitions of NR, NG-RAN, 5G Core Network (5GC) and network slice, to fulfil the requirements identified in 3GPP TS 28.540 [10].

The Information Model defines the semantics and behaviour of information object class attributes and relations visible on the management interfaces in a protocol and technology neutral way. And Solution Set defines one or more solution set(s) with specific protocol(s) according to the Information Model definitions.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System".

[3]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[4]	3GPP TS 38.401: "NG-RAN; Architecture description".

[5]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[6]	3GPP TS 38.420: "NG-RAN; Xn general aspects and principles".

[7]	3GPP TS 38.470: "NG-RAN; F1 general aspects and principles".

[8]	3GPP TS 38.473: "NG-RAN; F1 application protocol (F1AP)".

[9]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage 2".

[10]	3GPP TS 28.540: "Management and orchestration; 5G Network Resource Model (NRM);Stage 1".

[11]	3GPP TS 28.662: "Telecommunication management; Generic Radio Access Network (RAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS) ".

[12]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[13]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[14]	3GPP TS 36.410: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 general aspects and principles".

[15]	3GPP TS 36.423: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 application protocol".

[16]	3GPP TS 36.425: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 interface user plane protocol".

[17]	3GPP TS 28.625: "State Management Data Definition Integration Reference Point (IRP); Information Service (IS)".

[18]	ITU-T Recommendation X.731: "Information technology - Open Systems Interconnection - Systems Management: State management function".

[19]	3GPP TS 28.658: "Telecommunications management; Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)".

[20]	3GPP TS 28.702: "Core Network (CN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[21]	3GPP TS 28.708: "Telecommunication management; Evolved Packet Core (EPC) Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)".

[22]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[23]	3GPP TS 29.510: "5G system; Network Function Repository Services; Stage 3".

[24]	3GPP TS 29.531: "5G System; Network Slice Selection Services Stage 3".

[25]	Void.

[26]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[27]	3GPP TS 28.554: "Management and orchestration; 5G End to end Key Performance Indicators (KPI)".

[28]	3GPP TS 22.261: "Service requirements for next generation new services and markets".

[29]	ETSI GS NFV-IFA 013 V2.4.1 (2018-02) "Network Function Virtualisation (NFV); Management and Orchestration; Os-Ma-nfvo Reference Point - Interface and Information Model Specification".

[30]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[31]	Void.

[32]	3GPP TS 38.211: "NR; Physical channels and modulation".

[33]	3GPP TS 32.616: "Telecommunication management; Configuration Management (CM); Bulk CM Integration Reference Point (IRP); Solution Set (SS) definitions".

[34]	3GPP TS 28.623: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

[35]	3GPP TS 28.532: "Management and orchestration; Management services".

[36]	Void.

[37]	IETF RFC 791: "Internet Protocol".

[38]	IETF RFC 2373: "IP Version 6 Addressing Architecture".

[39]	IEEE 802.1Q: "Media Access Control Bridges and Virtual Bridged Local Area Networks".

[40]	ETSI GR NFV-IFA 015 (V2.4.1): "Network Function Virtualisation (NFV) Release 2; Management and Orchestration; Report on NFV Information Model".

[41]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[42]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[43]	3GPP TS 32.156: "Telecommunication management; Fixed Mobile Convergence (FMC) model repertoire".

[44]	IETF RFC 4122: "A Universally Unique IDentifier (UUID) URN Namespace".

[45]	IETF RFC 8528: "YANG Schema Mount".

[46]	Void

[47]	3GPP TS 32.160: "Management and orchestration; Management Service Template".

[48]	3GPP TS 38.463: "NG-RAN; E1 application protocol (E1AP)".

[49]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in Idle mode and RRC Inactive state".

[50]	GSMA NG.116 - Generic Network Slice Template Version 2.0 (2019-10-16).

[51]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains; Stage 1".

[52]	3GPP TS 33.501: " Security architecture and procedures for the 5G System".

[53]	3GPP TS 38.901: "Study on channel model for frequencies from 0.5 to 100 GHz ".

[54]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[55]	3GPP TS 38.215: "NR; Physical layer measurements".

[56]	3GPP TS 29.244: "Technical Specification Group Core Network and Terminals; Interface between the Control Plane and the User Plane Nodes; Stage 3".

[57]	3GPP TS 28.313: "Self-Organizing Networks (SON) for 5G networks".

[58]	3GPP TS 38.423: "NR; Xn application protocol (XnAP)".

[59]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[60]	3GPP TS 29.512: "5G System; Session Management Policy Control Service; Stage 3".

[61]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[62]	3GPP TS 29.214: "Policy and Charging Control over Rx reference point".

[63]	IETF RFC 7042: "IANA Considerations and IETF Protocol and Documentation Usage for IEEE 802 Parameters".

[64]	IEEE 802.3-2015: "IEEE Standard for Ethernet".

[65]	IEEE 802.1Q-2014: "Bridges and Bridged Networks".

[66]	IETF RFC 4301: "Security Architecture for the Internet Protocol".

[67]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[68]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management"

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1], TS 28.540 [10] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1] and TS 28.540 [10].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1], TS 23.501 [2], TS 38.401 [4], TS 28.540 [10] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1] , TS 23.501 [2], TS 38.401 [4] and TS 28.540 [10].

BWP	Bandwidth part

CM	Configuration Management

DN	Distinguished Name

IOC	Information Object Class

JSON	JavaScript Object Notation

NFV	Network Functions Virtualisation

NRM	Network Resource Model

NS	Network Service

NSI	Network Slice Instance

NSSAI	Network Slice Selection Assistance Information

NSSI	Network Slice Subnet Instance

PNF	Physical Network Function

RIM	Remote interference management

RIM-RS	Remote interference management reference signal

SBA	Service Based Architecture

SS	Solution Set

TN	Transport Network

VNF	Virtualised Network Function

# 4	Information model definitions for NR NRM

## 4.1	Imported and associated information

### 4.1.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.622 [30], IOC, ManagedFunction | ManagedFunction |
| TS 28.622 [30], IOC, EP_RP | EP_RP |
| TS 28.662 [11], IOC, SectorEquipmentFunction | SectorEquipmentFunction |
| TS 28.658 [19], IOC, ExternalENBFunction | ExternalENBFunction |
| TS 28.708 [21], IOC, ServingGWFunction | ServingGWFunction |
| TS 28.658 [19], IOC, EUtranCellFDD | EUtranCellFDD |
| TS 28.658 [19], IOC, EUtranCellTDD | EUtranCellTDD |
| TS 28.658 [19], dataType, PLMNId | PLMNId |
| TS 28.658 [19], IOC, ENBFunction | ENBFunction |
| TS 28.708 [21], IOC, ExternalServingGWFunction | ExternalServingGWFunction |
| TS 28.658 [19], IOC, ExternalEUtranCellFDD | ExternalEUtranCellFDD |
| TS 28.658 [19], IOC, ExternalEUtranCellTDD | ExternalEUtranCellTDD |
| TS 28.658 [19], IOC, AdjacentCell | AdjacentEUtranCell |
| TS 28.658 [19], IOC, EUtranFrequency | EUtranFrequency |
| TS 28.658 [19], IOC, EUtranFreqRelation | EUtranFreqRelation |
| TS 28.658 [19], IOC, EUtranRelation | EUtranCellRelation |

### 4.1.2	Associated information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.622 [30], IOC, ManagedElement | ManagedElement |
| TS 28.622 [30], IOC, SubNetwork | SubNetwork |

## 4.2	Class diagram

### 4.2.1	Class diagram for gNB and en-gNB

#### 4.2.1.1	Relationships

This clause depicts the set of classes (e.g. IOCs) that encapsulates the information relevant for this gNB and en-gNB. For the UML semantics, see TS 32.156 [43]. Subsequent clauses provide more detailed specification of various aspects of these classes.

The model fragments are for management representation of gNB and en-gNB for all NG-RAN deployment scenario as listed below.

-	Non-split NG-RAN deployment scenario, represents the gNB defined in TS 38.401[4]. In this scenario, a gNB is represented by a combination of a GNBCUCPFunction, one or more GNBCUUPFunctions and one or more GNBDUFunctions.

-	2-split NG-RAN deployment scenario, represents the gNB consist of gNB-CU and gNB-DU defined in TS 38.401[4] clause 6.1.1. In this scenario, a gNB-CU is represented by a combination of a GNBCUCPFunction and one or more GNBCUUPFunctions, whereas a gNB-DU is represented by a GNBDUFunction.

-	3-split NG-RAN deployment scenario, represents the gNB consist of gNB-CU-CP, gNB-CU-UP and gNB-DU defined in TS 38.401[4] clause 6.1.2. In this scenario, a gNB-CU-CP is represented by a GNBCUCPFunction, a gNB-CU-UP is represented by a GNBCUUPFunction, and a gNB-DU is represented by a GNBDUFunction.

Figure 4.2.1.1-1: NRM for all deployment scenarios

Figure 4.2.1.1-2: NRM for EPs for all deployment scenarios

Figure 4.2.1.1-3: NRM for <<IOC>>NRSectorCarrier and <<IOC>>BWP for all deployment scenarios

Figure 4.2.1.1-4: Cell Relation view for all deployment scenarios

NOTE 1:	The above NRM fragment uses SubNetwork to hold both NR and LTE external entities and frequencies.

Figure 4.2.1.1-5: Cell Relation view for all deployment scenarios

NOTE 2:	The above NRM fragment uses NRNetwork to hold NR external entities and frequency and using EUtraNetwork to hold LTE external entities and frequency. The NRNetwork and EUtraNetwork are subclasses of SubNetwork (defined in TS 28.622 [30]) with no additional attributes. The reason using NRNetwork and EUtraNetwork is for a clean separation of NR external entities and frequency and LTE external entities and frequency.

Figure 4.2.1.1-6: NRM fragment for RRM Policies

Figure 4.2.1.1-7: NRM fragment to support RIM

The Figure 4.2.1.1-8 shows the NRM fragment for configurable 5QIs in NG-RAN.

Figure 4.2.1.1-8: NRM fragment for configurable 5QIs in NG-RAN

Figure 4.2.1.1-9: NRM fragment for DANR Management

Figure 4.2.1.1-10: NRM fragment for DES Management

Figure 4.2.1.1-11: NRM fragment for DRACH Management

Figure 4.2.1.1-12: NRM fragment for DMRO Management

Figure 4.2.1.1-13: NRM fragment for DPCI Management

Figure 4.2.1.1-14: NRM fragment for CES Management

Figure 4.2.1.1-15: NRM fragment for CPCI Management

The Figure 4.2.1.1-16 shows the NRM fragment for dynamic 5QIs in NG-RAN.

Figure 4.2.1.1-16: NRM fragment for dynamic 5QIs in NG-RAN

#### 4.2.1.2	Inheritance

Figure 4.2.1.2-1: Inheritance Hierarchy

Figure 4.2.1.2-2: Inheritance Hierarchy

## 4.3	Class definitions

### 4.3.1	GNBDUFunction

#### 4.3.1.1	Definition

For non-split NG-RAN deployment scenario, this IOC together with GNBCUCPFunction IOC and GNBCUUPFunction IOC provide the management of gNB defined in clause 6.1.1 in TS 38.401 [4].

For 2-split and 3-split NG-RAN architecture, this IOC provides the management representation of gNB-DU defined in clause 6.1.1 in TS 38.401 [4].

The following table identifies the necessary end points required for the representation of gNB and en-gNB, of all deployment scenarios.

| Req Role | End point requirement for 3-split deployment scenario | End point requirement for 2-split deployment scenario | End point requirement for Non-split deployment scenario |
| --- | --- | --- | --- |
| gNB | <<IOC>>EP_F1C, <<IOC>>EP_F1U | <<IOC>>EP_F1C, <<IOC>>EP_F1U | None. |
| en-gNB | <<IOC>>EP_F1C, <<IOC>>EP_F1U | <<IOC>>EP_F1C, <<IOC>>EP_F1U | None. |

#### 4.3.1.2	Attributes

The GNBDUFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gNBDUId | M | T | T | F | T |
| gNBDUName | O | T | T | F | T |
| gNBId | M | T | F | F | T |
| gNBIdLength | M | T | T | F | T |
| rimRSReportConf | O | T | F | T | T |

#### 4.3.1.3	Attribute constraints

None.

#### 4.3.1.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.2	GNBCUCPFunction

#### 4.3.2.1	Definition

For non-split NG-RAN deployment scenario, this IOC together with GNBCUUPFunction IOC and GNBDUFunction IOC provide the management representation of gNB defined in clause 6.1.1 in TS 38.401 [4].

For 2-split NG-RAN deployment scenario, this IOC together with GNBCUUPFunction IOC provide management representation of the gNB-CU defined in clause 6.1.1 in TS 38.401 [4].

For 3-split NG-RAN deployment scenario, this IOC provides management representation of gNB-CU-CP defined in clause 6.1.2 in TS 38.401 [4].

The following table identifies the necessary end points required for the representation of gNB and en-gNB, of all deployment scenarios.

| Req Role | End point requirement for 3-split deployment scenario | End point requirement for 2-split deployment scenario | End point requirement for Non-split deployment scenario |
| --- | --- | --- | --- |
| gNB | <<IOC>>EP_XnC, <<IOC>>EP_NgC, <<IOC>>EP_F1C, <<IOC>>EP_E1. | <<IOC>>EP_XnC, <<IOC>>EP_NgC, <<IOC>>EP_F1C <<IOC>>EP_F1U. | <<IOC>>EP_XnC, <<IOC>>EP_NgC. |
| en-gNB | <<IOC>>EP_X2C, <<IOC>>EP_F1C, <<IOC>>EP_E1. | <<IOC>>EP_X2C, <<IOC>>EP_F1C. | <<IOC>>EP_X2C. |

#### 4.3.2.2	Attributes

The GNBCUCPFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gNBId | M | T | T | F | T |
| gNBIdLength | M | T | T | F | T |
| gNBCUName | O | T | T | F | T |
| pLMNId | M | T | T | T | T |
| x2BlackList | CM | T | T | F | T |
| x2WhiteList | CM | T | T | F | T |
| xnBlackList | M | T | T | F | T |
| xnWhiteList | M | T | T | F | T |
| x2XnHOBlackList | CM | T | T | F | T |
| mappingSetIDBackhaulAddressList | CM | T | T | F | T |
| tceIDMappingInfoList | CM | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| configurable5QISetRef | O | T | T | F | T |
| dynamic5QISetRef | O | T | F | F | T |

#### 4.3.2.3	Attribute constraints

| Name | Definition |
| --- | --- |
| x2BlackList | Condition: Multi-Radio Dual Connectivity with the EPC (see TS 37.340 [9] clause 4.1.2) is supported. |
| x2WhiteList | Condition: Multi-Radio Dual Connectivity with the EPC (see TS 37.340 [9] clause 4.1.2) is supported. |
| mappingSetIDBackhaulAddressList | Condition: Remote Interference Management function is supported. |
| tceIDMappingInfolist | Condition: MDT Function is supported. |

#### 4.3.2.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.3	GNBCUUPFunction

#### 4.3.3.1	Definition

For non-split NG-RAN deployment scenario, this IOC together with GNBCUCPFunction IOC and GNBDUFunction IOC provide the management representation of gNB defined in clause 6.1.1 in TS 38.401 [4].

For 2-split NG-RAN deployment scenario, this IOC together with GNBCUCPFunction IOC provide management representation of gNB-CU defined in clause 6.1.1 in TS 38.401 [4].

For 3-split NG-RAN deployment scenario, this IOC provides management representation of gNB-CU-UP defined in clause 6.1.2 in TS 38.401 [4].

The following table identifies the necessary end points required for the representation of gNB and en-gNB, of all deployment scenarios.

| Req Role | End point requirement for 3-split deployment scenario | End point requirement for 2-split deployment scenario | End point requirement for Non-split deployment scenario |
| --- | --- | --- | --- |
| gNB | <<IOC>>EP_XnU, <<IOC>>EP_NgU, <<IOC>>EP_F1U, <<IOC>>EP_E1. | <<IOC>>EP_XnU, <<IOC>>EP_NgU, <<IOC>>EP_F1U. | <<IOC>>EP_XnU, <<IOC>>EP_NgU. |
| en-gNB | <<IOC>>EP_X2U, <<IOC>>EP_S1U, <<IOC>>EP_F1U, <<IOC>>EP_E1. | <<IOC>>EP_X2U, <<IOC>>EP_S1U, <<IOC>>EP_F1U. | <<IOC>>EP_X2U, <<IOC>>EP_S1U. |

#### 4.3.3.2	Attributes

The GNBCUUPFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gNBCUUPId | M | T | F | T | T |
| pLMNInfoList | M | T | T | F | T |
| gNBId | M | T | T | F | T |
| gNBIdLength | M | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| configurable5QISetRef | O | T | T | F | T |
| dynamic5QISetRef | O | T | F | F | T |

#### 4.3.3.3	Attribute constraints

None.

#### 4.3.3.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.4	NRCellCU

#### 4.3.4.1	Definition

This IOC represents the part of NR cell information that is responsible for the management of inter-cell mobility and neighbour relations via ANR.

#### 4.3.4.2	Attributes

The NRCellCU IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cellLocalId | M | T | T | F | T |
| pLMNInfoList | M | T | T (Note) | F | T |
| Attribute related to role |  |  |  |  |  |
| nRFrequencyRef | M | T | F | F | T |
| Note: Whether the attribute "pLMNId" in the PLMNInfo can be writable depends on the implementation. |  |  |  |  |  |

Note 1:	Void.

Note 2:	Void.

#### 4.3.4.3	Void

#### 4.3.4.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.5	NRCellDU

#### 4.3.5.1	Definition

This IOC represents the part of NR cell information that describes s the specific resources instances.

An NR cell transmits SS/PBCH block and always requires downlink transmission at a certain carrier frequency with a certain channel bandwidth. Transmission may be performed from multiple sector-carriers using different transmission points, and these may be configured with different carrier frequencies and channel bandwidths, as long as they are aligned to the cell's downlink resource grids as defined in clause 4.4 in TS 38.211 [32]. The values of arfcnDL and bSChannelBwDL attributes define the resource grids which each sector-carrier needs to be aligned to. See clauses 5.3 and 5.4.2 of TS 38.104 for definitions of BS channel bandwidth and NR-ARFCN, respectively.

An NR cell requires an uplink in order to provide initial access. In case of TDD, the values of arfcnUL and bSChannelBwUL have to always be set to the same values as for the corresponding DL attributes. For both FDD and TDD, the arfcnUL and bSChannelBwUL define uplink resource grids to which each sector-carrier needs to align to.

An NR cell can in addition be configured with a supplementary uplink, which has its own arfcnSUL and bSChannelBwSUL, which define resource grids for supplementary uplink sector-carriers.

Each of downlink, uplink and supplementary uplink (if configured) need an initial bandwidth part (BWP), which defines resources to be used by UEs during and immediately after initial access. Additional BWPs can be either configured or calculated by gNB internally and be applied to UEs dynamically by gNB based on e.g. UE capability and bandwidth need of each UE.

NOTE: Void

#### 4.3.5.2	Attributes

The NRCellDU IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cellLocalId | M | T | T | F | T |
| operationalState | M | T | F | F | T |
| administrativeState | M | T | T | F | T |
| cellState | M | T | F | F | T |
| pLMNInfoList | M | T | T | F | T |
| nRPCI | M | T | T | F | T |
| nRTAC | CM | T | T | F | T |
| arfcnDL | M | T | T | F | T |
| arfcnUL | CM | T | T | F | T |
| arfcnSUL | CM | T | T | F | T |
| bSChannelBwDL | M | T | T | F | T |
| ssbFrequency | CM | T | T | F | T |
| ssbPeriodicity | M | T | T | F | T |
| ssbSubCarrierSpacing | CM | T | T | F | T |
| ssbOffset | M | T | T | F | T |
| ssbDuration | M | T | T | F | T |
| bSChannelBwUL | CM | T | T | F | T |
| bSChannelBwSUL | CM | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| nRSectorCarrierRef | M | T | T | F | T |
| bWPRef | M | T | T | F | T |
| nRFrequencyRef | CO | T | T | F | T |
| victimSetRef | CM | T | T | F | T |
| aggressorSetRef | O | T | T | F | T |
| Note 1: No state propagation is implied. Note 2: Void |  |  |  |  |  |

#### 4.3.5.3	Attribute constraints

| Name | Definition |
| --- | --- |
| arfcnUL Support Qualifier | Condition: The cell has an uplink (FDD or TDD) |
| arfcnSUL Support Qualifier | Condition: The cell has a supplementary uplink |
| bSChannelBwUL Support Qualifier | Condition: The cell has an uplink (FDD or TDD) |
| bSChannelBwSUL Support Qualifier | Condition: The cell has a supplementary uplink |
| nRFrequencyRef Support Qualifier | Condition: Non-split deployment scenario is supported |
| ssbFrequency Support Qualifier | Condition: nRFrequencyRef is not used. |
| ssbSubCarrierSpacing Support Qualifier | Condition: nRFrequencyRef is not used. |
| victimSetRef Support Qualifier | Condition: RIM feature is supported |

#### 4.3.5.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.6	NRSectorCarrier

#### 4.3.6.1	Definition

This <<IOC>>NRSectorCarrier represents the resources of each transmission point associated to corresponding cell(s). These in general have different physical locations (of the antennae), and possibly different frequencies or bandwidths. The UE is not directly aware of which NRSectorCarrier resources the network uses for its connection.

An NR sector-carrier can have downlink, uplink or both as specified by txDirection. Attributes related to unavailable direction (DL or UL) shall not be set.

Additional NRSectorCarriers not directly associated to one cell only can also be configured.

If a value of arfcnDL, arfcnUL, bSChannelBwDL or bSChannelBwUL can be derived unambiguously from the referring cell, then that attribute needs not be present. That will not be possible if the NRSectorCarrier is used for supplementary uplink, if it is not directly associated to a cell, or if the sector-carrier uses only a part of the cell's channel bandwidth. Thus, at least in those cases the applicable attributes have to be present and their values need to be set.

#### 4.3.6.2	Attributes

The NRSectorCarrier IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| txDirection | M | T | T | F | T |
| configuredMaxTxPower | CM | T | T | F | T |
| configuredMaxTxEIRP | CM | T | T | F | T |
| arfcnDL | CM | T | T | F | T |
| arfcnUL | CM | T | T | F | T |
| bSChannelBwDL | CM | T | T | F | T |
| bSChannelBwUL | CM | T | T | F | T |
| attribute related to role |  |  |  |  |  |
| sectorEquipmentFunctionRef | M | T | T | F | T |

#### 4.3.6.3	Attribute constraints

| Name | Definition |
| --- | --- |
| configuredMaxTxPower | Condition: The sector-carrier has a downlink. Configuration of Tx power at antenna port reference point is supported. |
| configuredMaxTxEIRP | Condition: The sector-carrier has a downlink. Configuration of emitted isotropic radiated power is supported. |
| arfcnDL | Condition: The sector-carrier has a downlink AND the value differs from the referring cell's value of arfcnDL. |
| arfcnUL | Condition: The sector-carrier has an uplink AND the value differs from the referring cell's value of arfcnUL. |
| bSChannelBwDL | Condition: The sector-carrier has a downlink AND the value differs from the referring cell's value of bSChannelBwDL. |
| bSChannelBwUL | Condition: The sector-carrier has an uplink AND the value differs from the referring cell's value of bSChannelBwUL. |

#### 4.3.6.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.7	BWP

#### 4.3.7.1	Definition

This IOC represents a bandwidth part (BWP) defined in TS 38.211 [32], clause 4.4.5. A bandwidth part is related to downlink, uplink or supplementary uplink resource grids, and is defined by its subcarrier spacing (SCS), cyclic prefix and location and size related to the common resource grid for the applicable SCS.

A BWP can be either an initial BWP used for initial access, or other ("regular") BWP configured for relevant UEs that support the BWP's characteristics.

#### 4.3.7.2	Attributes

The BWP IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| bwpContext | M | T | T | F | T |
| isInitialBwp | M | T | T | F | T |
| subCarrierSpacing | M | T | T | F | T |
| cyclicPrefix | M | T | T | F | T |
| startRB | M | T | T | F | T |
| numberOfRBs | M | T | T | F | T |

#### 4.3.7.3	Attribute constraints

None.

#### 4.3.7.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.8	EP_E1

#### 4.3.8.1	Definition

This IOC represents the local end point of the logical link, supporting E1 interface between gNB-CU-CP and gNB-CU-UP. The E1 interface is defined in TS 38.401 [4].

#### 4.3.8.2	Attributes

The EP_E1 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.8.3	Attribute constraints

None.

#### 4.3.8.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.9	EP_XnU

#### 4.3.9.1	Definition

This IOC represents the one end-point of a logical link supporting the Xn user plane (Xn-U) interface. The Xn-U interface provides non-guaranteed delivery of user plane PDUs between two NG-RAN nodes. The user plane PDUs are carried on GTP-U/UDP/IP/Data link layer/Physical layer stack. See clause 7.2 of TS 38.420 [6].

#### 4.3.9.2	Attributes

The EP_XnU IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.9.3	Attribute constraints

None.

#### 4.3.9.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.10	EP_NgC

#### 4.3.10.1	Definition

This IOC represents the local end point of the control plane interface (NG-C) between the gNB and NG-Core entity. The transport network layer is built on IP transport. For the reliable transport of signalling messages, SCTP is added on top of IP. The application layer signalling protocol is referred to as NG-AP (NG Application Protocol).

TS 38.470 [7] noted that "one gNB-CU and a set of gNB-DUs are visible to other logical nodes as a gNB or an en-gNB where the gNB terminates the Xn and the NG interfaces, and the en-gNB terminates the X2 and the S1-U interfaces".

#### 4.3.10.2	Attributes

The EP_NgC IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.10.3	Attribute constraints

None.

#### 4.3.10.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.11	EP_NgU

#### 4.3.11.1	Definition

This IOC represents the local end point of the NG user plane (NG-U) interface between the gNB and the UPGW. The interface provides non-guaranteed delivery of user plane PDUs between the gNB and the UPGW. GTP-U is baseline for this interface.

TS 38.470 [7] noted that "one gNB-CU and a set of gNB-DUs are visible to other logical nodes as a gNB or an en-gNB where the gNB terminates the Xn and the NG interfaces, and the en-gNB terminates the X2 and the S1-U interfaces".

#### 4.3.11.2	Attributes

The EP_NgU IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| epTransportRef | O | T | F | F | T |

#### 4.3.11.3	Attribute constraints

None.

#### 4.3.11.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.12	EP_F1C

#### 4.3.12.1	Definition

This IOC represents the local end point of the control plane interface (F1-C) between the DU and CU or CU-CP. The transport network layer is based on IP transport with the SCTP on top of IP. The application layer signalling protocol is referred to as NG-AP (NG Application Protocol). See clause 7.1 of TS 38.470 [7].

TS 38.470 [7] noted that "one gNB-CU and a set of gNB-DUs are visible to other logical nodes as a gNB or an en-gNB where the gNB terminates the Xn and the NG interfaces, and the en-gNB terminates the X2 and the S1-U interfaces".

#### 4.3.12.2	Attributes

The EP_F1C IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.12.3	Attribute constraints

None.

#### 4.3.12.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.13	EP_F1U

#### 4.3.13.1	Definition

This IOC represents the local end point of the user plane interface (F1-U) between the DU and CU or CU-UP. The transport network layer is based on IP transport, with the UDP and GTP-U on top of IP.

TS 38.470 [7] noted that "one gNB-CU and a set of gNB-DUs are visible to other logical nodes as a gNB or an en-gNB where the gNB terminates the Xn and the NG interfaces, and the en-gNB terminates the X2 and the S1-U interfaces".

#### 4.3.13.2	Attributes

The EP_F1U IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.13.3	Attribute constraints

None.

#### 4.3.13.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.14	EP_S1U

#### 4.3.14.1	Definition

This IOC represents the local end point of the logical link, supporting S1-U interface towards a S-GW node. The S1-U interface is defined in TS 36.410 [14].

#### 4.3.14.2	Attributes

The EP_S1U IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.14.3	Attribute constraints

None.

#### 4.3.14.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.15	EP_X2C

#### 4.3.15.1	Definition

This IOC represents the local end point of the logical link, supporting X2-C application protocols used in EN-DC, to a neighbour eNB or en-gNB node, which is defined in TS 36.423 [15]. EN-DC is defined in TS 37.340 [9].

#### 4.3.15.2	Attributes

The EP_X2C IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.15.3	Attribute constraints

None.

#### 4.3.15.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.16	EP_X2U

#### 4.3.16.1	Definition

This IOC represents the local end-point of a logical link supporting the X2 user plane (X2-U) interface used in EN-DC, which is defined in TS 36.425 [16].

#### 4.3.16.2	Attributes

The EP_X2U IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.16.3	Attribute constraints

None.

#### 4.3.16.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.17	EP_XnC

#### 4.3.17.1	Definition

This IOC represents the local gNB node end point of the logical link, supporting Xn Application protocols, to a neighbour NG-RAN node (including gNB and ng-eNB). The Xn Application PDUs are carried over SCTP/IP/Data link layer/Physical layer stack. See clause 7 of TS 38.420 [6].

#### 4.3.17.2	Attributes

The EP_XnC IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.17.3	Attribute constraints

None

#### 4.3.17.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.18	ExternalGNBCUCPFunction

#### 4.3.18.1	Definition

This IOC represents the properties, known by the management function, of a GNBCUCPFunction managed by another management function. For more information about GNBCUCPFunction, see clause 4.3.2.

#### 4.3.18.2	Attributes

The ExternalGNBCUCPFunction includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gNBId | M | T | T | F | T |
| gNBIdLength | M | T | T | F | T |
| pLMNId | M | T | T | F | T |

#### 4.3.18.3	Attribute constraints

None.

#### 4.3.18.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.19	ExternalGNBCUUPFunction

#### 4.3.19.1	Definition

This IOC represents the properties, known by the management function, of a GNBCUUPFunction managed by another management function. For more information about GNBCUUPFunction, see clause 4.3.3.

#### 4.3.19.2	Attributes

The ExternalGNBCUUPFunction includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gNBId | M | T | T | F | T |
| gNBIdLength | M | T | T | F | T |

#### 4.3.19.3	Attribute constraints

None.

#### 4.3.19.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.20	ExternalGNBDUFunction

#### 4.3.20.1	Definition

This IOC represents the properties, known by the management function, of a GNBDUFunction managed by another management function. For more information about GNBDUFunction, see clause 4.3.1.

#### 4.3.20.2	Attributes

The ExternalGNBDUFunction includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gNBId | M | T | T | T | T |
| gNBIdLength | M | T | T | F | T |

#### 4.3.20.3	Attribute constraints

None.

#### 4.3.20.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.21	ExternalUPFFunction

#### 4.3.21.1	Definition

This IOC represents the properties, known by the management function, of a UPFFunction managed by another management function. For more information about UPFFunction, see clause 5.3.3.

#### 4.3.21.2	Attributes

The ExternalUPFFunction includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |

#### 4.3.21.3	Attribute constraints

None.

#### 4.3.21.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.22	ExternalAMFFunction

#### 4.3.22.1	Definition

This IOC represents the properties, known by the management function, of an AMFFunction managed by another management function. For more information about AMFFunction, see clause 5.3.

#### 4.3.22.2	Attributes

The ExternalAMFFunction includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |

#### 4.3.22.3	Attribute constraints

None

#### 4.3.22.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.23	Void

### 4.3.24	ENBFunction <<ProxyClass>>

#### 4.3.24.1	Definition

This IOC represents an <<IOC>>ENBFunction and <<IOC>>ExternalENBFunction.

#### 4.3.24.2	Attributes

See that defined in <<IOC>>ENBFunction and <<IOC>>ExternalENBFunction.

#### 4.3.24.3	Attribute constraints

See that defined in <<IOC>>ENBFunction and <<IOC>>ExternalENBFunction.

#### 4.3.24.4	Notifications

See respective IOCs.

### 4.3.25	GNBCUCPFunction <<ProxyClass>>

#### 4.3.25.1	Definition

This IOC represents an <<IOC>>GNBCUCPFunction and <<IOC>>ExternalGNBCUCPFunction.

#### 4.3.25.2	Attributes

See that defined in <<IOC>>GNBCUCPFunction and <<IOC>>ExternalGNBCUCPFunction.

#### 4.3.25.3	Attribute constraints

See respective IOCs.

#### 4.3.25.4	Notifications

See respective IOCs.

### 4.3.26	GNBCUUPFunction <<ProxyClass>>

#### 4.3.26.1	Definition

This IOC represents an <<IOC>>GNBCUUPFunction and <<IOC>>ExternalGNBCUUPFunction.

#### 4.3.26.2	Attributes

See that defined in <<IOC>>GNBCUUPFunction and <<IOC>>ExternalGNBCUUPFunction.

#### 4.3.26.3	Attribute constraints

See that defined in <<IOC>>GNBCUUPFunction and <<IOC>>ExternalGNBCUUPFunction.

#### 4.3.26.4	Notifications

See respective IOCs.

### 4.3.27	GNBDUFunction <<ProxyClass>>

#### 4.3.27.1	Definition

This IOC represents an <<IOC>>GNBDUFunction and <<IOC>>ExternalGNBDUFunction.

#### 4.3.27.2	Attributes

See that defined in <<IOC>>GNBDUFunction and <<IOC>>ExternalGNBDUFunction.

#### 4.3.27.3	Attribute constraints

See that defined in <<IOC>>GNBDUFunction and <<IOC>>ExternalGNBDUFunction.

#### 4.3.27.4	Notifications

See respective IOCs.

### 4.3.28	ServingGWFFunction <<ProxyClass>>

#### 4.3.28.1	Definition

This IOC represents an <<IOC>>ServingGWFFunction and <<IOC>>ExternalServingGWFunction.

#### 4.3.28.2	Attributes

See that defined in <<IOC>>ServingGWFunction and <<IOC>>ExternalServingGWFunction.

#### 4.3.28.3	Attribute constraints

See that defined in <<IOC>>ServingGWFunction and <<IOC>>ExternalServingGWFunction.

#### 4.3.28.4	Notifications

See respective IOCs.

### 4.3.29	UPFFunction <<ProxyClass>>

#### 4.3.29.1	Definition

This IOC represents an <<IOC>>UPFFunction and <<IOC>>ExternalUPFFunction.

#### 4.3.29.2	Attributes

See that defined in <<IOC>>UPFFunction and <<IOC>>ExternalUPFFunction.

#### 4.3.29.3	Attribute constraints

See that defined in <<IOC>>UPFFunction and <<IOC>>ExternalUPFFunction.

#### 4.3.29.4	Notifications

See respective IOCs.

### 4.3.30	AMFFunction <<ProxyClass>>

#### 4.3.30.1	Definition

This IOC represents an <<IOC>>AMFFunction and <<IOC>>ExternalAMFFunction.

#### 4.3.30.2	Attributes

See that defined in <<IOC>>AMFFunction and <<IOC>>ExternalAMFFunction.

#### 4.3.30.3	Attribute constraints

See that defined in <<IOC>>AMFFunction and <<IOC>>ExternalAMFFunction.

#### 4.3.30.4	Notifications

See respective IOCs.

### 4.3.31	Void

### 4.3.32	NRCellRelation

#### 4.3.32.1	Definition

This IOC represents a neighbour cell relation from a source cell to a target cell, where the target cell is an NRCellCU or ExternalNRCellCU instance.

The source cell can be a NRCellCU instance. This is the case for an Intra-NR neighbour cell relation.

The source cell can be a EUtranGenericCell instance. This is the case for Inter-LTE-NR neighbour cell relation, from E-UTRAN to NR. See TS 28.658 [19].

Neighbour cell relations are unidirectional.

#### 4.3.32.2	Attributes

| Attribute name |  | Support Qualifier |  | isReadable |  | isWritable |  | isInvariant |  | isNotifyable |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| nRTCI |  | O |  | T |  | T |  | F |  | T |  |
| cellIndividualOffset |  | M |  | T |  | T |  | F |  | T |  |
| isRemoveAllowed |  | CM |  | T |  | T |  | F |  | T |  |
| isHOAllowed |  | CM |  | T |  | T |  | F |  | T |  |
| isESCoveredBy |  | CM |  | T |  | T |  | F |  | T |  |
| isENDCAllowed |  | CM |  | T |  | T |  | F |  | T |  |
| attribute related to role |  |  |  |  |  |  |  |  |  |  |  |
| nRFreqRelationRef |  | M |  | T |  | T |  | F |  | T |  |
| adjacentNRCellRef |  | M |  | T |  | T |  | F |  | T |  |

#### 4.3.32.3	Attribute constraints

| Name | Definition |
| --- | --- |
| isRemoveAllowed | Condition: ANR function is supported in the source cell. |
| isHOAllowed | Condition: ANR function is supported in the source cell. |
| isESCoveredBy | Condition: Energy Saving function is supported. |
| isENDCAllowed | Condition: Multi-Radio Dual Connectivity with the EPC (see TS 37.340 [9] clause 4.1.2) is supported. |

#### 4.3.32.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.33	NRFreqRelation

#### 4.3.33.1	Definition

This IOC, together with the target NRFrequency, represents the frequency properties applicable to the referencing NRCellRelation.

#### 4.3.33.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| offsetMO | O | T | T | F | F |
| blackListEntry | O | T | T | F | F |
| blackListEntryIdleMode | O | T | T | F | F |
| cellReselectionPriority | O | T | T | F | F |
| cellReselectionSubPriority | O | T | T | F | F |
| pMax | O | T | T | F | F |
| qOffsetFreq | O | T | T | F | F |
| qQualMin | O | T | T | F | F |
| qRxLevMin | M | T | T | F | F |
| threshXHighP | M | T | T | F | F |
| threshXHighQ | CM | T | T | F | F |
| threshXLowP | M | T | T | F | F |
| threshXLowQ | CM | T | T | F | F |
| tReselectionNr | M | T | T | F | F |
| tReselectionNRSfHigh | O | T | T | F | F |
| tReselectionNRSfMedium | O | T | T | F | F |
| attribute related to role |  |  |  |  |  |
| nRFrequencyRef | M | T | T | F | F |

#### 4.3.33.3	Attribute constraints

| Name | Definition |
| --- | --- |
| threshXHighQ | Condition: RSRQ used in SIB4. |
| threshXLowQ | Condition: RSRQ used in SIB4. |

#### 4.3.33.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.34	Void

### 4.3.35	ExternalNRCellCU

#### 4.3.35.1	Definition

This abstract IOC represents the properties of an NRCellCU controlled by another Management Service Provider. This IOC contains necessary attributes for inter-system and intra-system handover. It also contains a subset of the attributes of related IOCs controlled by Management Service Provider. The way to maintain consistency between the attribute values of these IOCs is outside the scope of the present document.

#### 4.3.35.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cellLocalId | M | T | T | F | T |
| nRPCI | M | T | T | F | T |
| plmnIdList | M | T | T | F | T |
| attribute related to role |  |  |  |  |  |
| nRFrequencyRef | M | T | T | F | T |

#### 4.3.35.3	Attribute constraints

None.

#### 4.3.35.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.36	RRMPolicyRatio

#### 4.3.36.1	Definition

This IOC represents the properties of RRMPolicyRatio. RRMPolicyRatio is one realization of abstract RRMPolicy_ IOC. RRMPolicyRatio has three attributes, apart from those inherited (DN, resourceType, rRMPolicyMemberList).

Figure 4.3.36-1 Structure of RRMPolicyRatio

-	The attribute rRMPolicyMaxRatio defines the maximum resource usage quota for the associated rRMPolicyMemberList, including at least one of shared resources, prioritized resources and dedicated resources. The sum of the 'rRMPolicyMaxRatio' values assigned to all RRMPolicyRatio(s) name-contained by same MangedEntity can be greater than 100.

-	The attribute rRMPolicyMinRatio defines the minimum resource usage quota for the associated RRMPolicyMemberList, including at least one of prioritized resources and dedicated resources, which means the resources quota that need to be guaranteed for use by the associated rRMPolicyMemberList. The sum of the 'rRMPolicyMinRatio' values assigned to all RRMPolicyRatio(s) name-contained by same MangedEntity shall be less or equal 100.

-	The attribute rRMPolicyDedicatedRatio defines the dedicated resource usage quota for the RRMPolicyMemberList, including dedicated resources. The sum of the 'rRMPolicyDedicatedRatio' values assigned to all RRMPolicyRatio(s) name-contained by same MangedEntity shall be less or equal 100.

The following are the definition for above mentioned three resource categories:

-	Shared resources: means the resources that are shared with other rRMPolicyMemberList(s) (i.e. the rRMPolicyMemberList(s) defined in RRMPolicyRatio(s) name-contained by the same ManagedEntity). The shared resources are not guaranteed for use by the associated rRMPolicyMemberList. The shared resources quota is represented by [rRMPolicyMaxRatio-rRMPolicyMinRatio].

-	Priortized resources: means the resources are preferentially used by the associated RRMPolicyMemberList. These resources are guaranteed for use by the associated RRMPolicyMemberList when it needs to use them. When not used, these resources may be used by other rRMPolicyMemberList(s) (i.e. the rRMPolicyMemberList(s) defined in RRMPolicyRatio(s) name-contained by the same ManagedEntity). The prioritized resources quota is represented by [rRMPolicyMinRatio-rRMPolicyDedicatedRatio]

-	Dedicated resources: means the resources are dedicated for use by the associated RRMPolicyMemberList. These resources can not be shared even if the associated RRMPolicyMember does not use them. The Dedicated resources quota is represented by [rRMPolicyDedicatedRatio].

#### 4.3.36.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| rRMPolicyMaxRatio | M | T | T | F | T |
| rRMPolicyMinRatio | M | T | T | F | T |
| rRMPolicyDedicatedRatio | O | T | T | F | T |

#### 4.3.36.3	Attribute constraints

None

#### 4.3.36.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.37	S-NSSAI <<dataType>>

#### 4.3.37.1	Definition

This data type represents an S-NSSAI. An NSSAI is a set of supported S-NSSAI(s), an S-NSSAI is comprised of an SST (Slice/Service type) and an optional SD (Slice Differentiator) field, (See TS 23.003 [13]).

#### 4.3.37.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| sST | M | T | T | F | T |
| sD | O | T | T | F | T |

#### 4.3.37.3	Attribute constraints

None

#### 4.3.37.4	Notifications

The clause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.38	NRFrequency

#### 4.3.38.1	Definition

This IOC represents certain NR frequency properties.

#### 4.3.38.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| absoluteFrequencySSB | M | T | T | F | T |
| sSBSubCarrierSpacing | M | T | T | F | T |
| multiFrequencyBandListNR | O | T | F | F | T |

#### 4.3.38.3	Attribute constraints

None.

#### 4.3.38.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.39	CommonBeamformingFunction

#### 4.3.39.1	Definition

This <<IOC>>CommonBeamformingFunction represents common beamforming functionality (eg: SSB beams) for the NRSectorCarrier.

The CommonBeamformingFunction provides capability to configure the advanced antenna for a sector carrier. The configuration capability is provided by selection of coverageShape, digitalTilt and digitalAzimuth. These attributes represent the wanted coverage area and radiation pattern on a sector carrier related to an antenna transmission point.

This configuration capability assumes the system shall handle configuration of SSB beams within the sector carrier. Individual SSB beams within a sector carrier cannot be independently configured as this depends on many conditions and constraints, for instance TDD patterns, allocations of PRACH occasions, SIB1 and mobility considerations.

The associated <<IOC>> Beam provides information beam direction and beam width for the associated SSB beams as a result of the configuration. The beams addressed in this definition are the common beams.  There may be more than one beam per CommonBeamformingFunction for the NRSectorCarrier.

#### 4.3.39.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| coverageShape | M | T | T | F | T |
| digitalTilt | M | T | T | F | T |
| digitalAzimuth | M | T | T | F | T |

#### 4.3.39.3	Attribute constraints

None.

#### 4.3.39.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.40	Beam

#### 4.3.40.1	Definition

This <<IOC>>Beam represents the per-Beam information required for, e.g. beam performance management utilizing measurements generated in the RAN. TS 38.104 [12] relates to beam transmission, TS 38.215 [55] to beam measurements, and TS 38.331 [54] to reporting of those measurements and associated beam failure Information Elements, clauses 5.5.3, 5.5.5.2, 6.3.2. 6.2.2.

Measurements on common beams may be correlated with associated spatial beam information to assist use cases like troubleshooting performance problems, or SON functions like Coverage & Capacity Optimization.

<<IOC>>Beam can have spatial attributes of horizontal/azimuth (ie: Phi φ-axis) and vertical/tilt (ie: Theta θ-axis) beam pointing direction and beam width attributes.  There may be more than one beam per CommonBeamformingFunction for an NRSectorCarrier.  Informational note, beam direction and width are characteristics—a representation—of directional energy vectors.

#### 4.3.40.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| beamIndex | M | T | F | F | T |
| beamType | O | T | F | F | T |
| beamAzimuth | CM | T | F | F | T |
| beamTilt | CM | T | F | F | T |
| beamHorizWidth | CM | T | F | F | T |
| beamVertWidth | CM | T | F | F | T |

#### 4.3.40.3	Attribute constraints

| Name | Definition |
| --- | --- |
| beamAzimuth Support Qualifier | Condition: The beamType is "SSB-BEAM" and Supported by Equipment |
| beamTilt Support Qualifier | Condition: The beamType is "SSB-BEAM" and Supported by Equipment |
| beamHorizWidth Support Qualifier | Condition: The beamType is "SSB-BEAM" and Supported by Equipment |
| beamVertWidth Support Qualifier | Condition: The beamType is "SSB-BEAM" and Supported by Equipment |

### 4.3.41	PLMNInfo <<dataType>>

#### 4.3.41.1	Definition

This <<dataType>> represents the PLMN supported by the <<IOC>> using this <<dataType>> as one of its attributes. In case of network slicing feature is supported, this <<dateType>> also represents the S-NSSAI in the PLMN supported by the <<IOC>> using this <<dataType>> as one of its attributes.

#### 4.3.41.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNId | M | T | T | F | T |
| sNSSAI | CM | T | T | F | T |

#### 4.3.41.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAI Support Qualifier | Condition: Network slicing feature is supported. |

#### 4.3.41.4	Notifications

The <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.42	RRMPolicyMember <<dataType>>

#### 4.3.42.1	Definition

This <<dataType>> represents an RRM Policy member that will be part of a rRMPolicyMemberList. A RRMPolicyMember is defined by its pLMNId and sNSSAI (S-NSSAI). The members in a rRMPolicyMemberList is assigned a specific amount of RRM resources based on settings in RRMPolicy_.

#### 4.3.42.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNId | M | T | T | F | T |
| sNSSAI | CM | T | T | F | T |

#### 4.3.42.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAI Support Qualifier | Condition: Network slicing is supported |

#### 4.3.42.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.43	RRMPolicy_

#### 4.3.43.1	Definition

This IOC represents the properties of an abstract RRMPolicy. The RRMPolicy_ IOC needs to be subclassed to be instantiated. It defines two attributes apart from those inherited from TOP IOC, the resourceType attribute defines type of resource (PRB, RRC connected users, DRB usage etc.) and the rRMPolicyMemberList attribute defines the RRMPolicyMember(s)that is subject to this policy. An RRM resource (defined in resourceType attribute) is located in NRCellDU, NRCellCU, GNBDUFunction, GNBCUCPFunction or in GNBCUUPFunction. The RRMPolicyRatio IOC is one realization of a RRMPolicy_ IOC, see the inheritance in Figure 4.2.1.2-1. This RRM framework allows adding new policies, both standardized or as vendor specific, by inheriting from the abstract RRMPolicy_ IOC.

#### 4.3.43.2	Attributes

The RRMPolicy_ IOC have the following attributes, apart from those inherited from TOP IOC (defined in TS 28.622 [30]):

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| resourceType | M | T | T | F | T |
| rRMPolicyMemberList | M | T | T | F | T |

#### 4.3.43.3	Attribute constraints

None.

#### 4.3.43.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.44	RRMPolicyManagedEntity <<ProxyClass>>

#### 4.3.44.1	Definition

This represents an <<IOC>>NRCellCU, or an <<IOC>>NRCellDU or an <<IOC>>GNBCUUPFunction, or an <<IOC>>GNBCUCPFunction, or an <<IOC>>GNBDUFunction.

If <<IOC>>NRCellCU is used, which means that a RRMPolicy shall be applied to an RRM resource in the NRCellCU. The possible RRM resource(s) owned by NRCellCU is defined in the resourceType attribute.

If <<IOC>>NRCellDU is used, which means that a RRMPolicy shall be applied to an RRM resource in the NRCellDU. The possible RRM resource(s) owned by NRCellDU is defined in the resourceType attribute.

If <<IOC>>GNBCUUPFunction is used, which means that a RRMPolicy shall be applied to an RRM resource in the GNBCUUPFunction. The possible RRM resource(s) owned by GNBCUUPFunction is defined in the resourceType attribute.

If <<IOC>>GNBCUCPFunction is used, which means that a RRMPolicy shall be applied to an RRM resource in the GNBCUCPFunction. The possible RRM resource(s) owned by GNBCUCPFunction is defined in the resourceType attribute.

If <<IOC>>GNBDUFunction is used, which means that a RRMPolicy shall be applied to an RRM resource in the GNBDUFunction. The possible RRM resource(s) owned by GNBDUFunction is defined in the resourceType attribute.

#### 4.3.44.2	Attributes

See that defined in <<IOC>>NRCellCU, <<IOC>>NRCellDU, <<IOC>>GNBCUUPFunction, <<IOC>>GNBCUCPFunction or <<IOC>>GNBDUFunction.

#### 4.3.44.3	Attribute constraints

See that defined in <<IOC>>NRCellCU, <<IOC>>NRCellDU, <<IOC>>GNBCUUPFunction, <<IOC>>GNBCUCPFunction, or <<IOC>>GNBDUFunction.

#### 4.3.44.4	Notifications

See respective IOCs.

### 4.3.45	GNBCUCPNeighbour <<ProxyClass>>

#### 4.3.45.1	Definition

This IOC represents an <<IOC>>GNBCUCPFunction, <<IOC>>ExternalGNBCUCPFunction, <<IOC>>ENBFunction and <<IOC>>ExternalENBFunction.

#### 4.3.45.2	Attributes

See that defined in <<IOC>>GNBCUCPFunction, <<IOC>>ExternalGNBCUCPFunction, <<IOC>>ENBFunction and <<IOC>>ExternalENBFunction.

#### 4.3.45.3	Attribute constraints

See that defined in <<IOC>>GNBCUCPFunction, <<IOC>>ExternalGNBCUCPFunction, <<IOC>>ENBFunction and <<IOC>>ExternalENBFunction.

#### 4.3.45.4	Notifications

See respective IOCs.

### 4.3.46	GNBCUUPNeighbour <<ProxyClass>>

#### 4.3.46.1	Definition

This IOC represents an <<IOC>>GNBCUUPFunction, <<IOC>>ExternalGNBCUUPFunction, <<IOC>>ENBFunction and <<IOC>>ExternalENBFunction.

#### 4.3.46.2	Attributes

See that defined in <<IOC>>GNBCUUPFunction, <<IOC>>ExternalGNBCUUPFunction, <<IOC>>ENBFunction and <<IOC>>ExternalENBFunction.

#### 4.3.46.3	Attribute constraints

See that defined in <<IOC>>GNBCUUPFunction, <<IOC>>ExternalGNBCUUPFunction, <<IOC>>ENBFunction and <<IOC>>ExternalENBFunction.

#### 4.3.46.4	Notifications

See respective IOCs.

### 4.3.47	MappingSetIDBackhaulAddress  <<dataType>>

#### 4.3.47.1	Definition

This data type represents the properties describing the mapping relationship between set ID and backhaul address of gNB.

#### 4.3.47.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| setID | M | T | T | F | T |
| backhaulAddress | M | T | T | F | T |

#### 4.3.47.3	Attribute constraints

None.

#### 4.3.47.4	Notifications

The clause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.48	BackhaulAddress  <<dataType>>

#### 4.3.48.1	Definition

This data type represents the properties describing the backhaul address of gNB.

#### 4.3.48.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gNBId | M | T | T | F | T |
| tAI | M | T | T | F | T |

#### 4.3.48.3	Attribute constraints

None.

#### 4.3.48.4	Notifications

The clause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.49	TAI  <<dataType>>

#### 4.3.49.1	Definition

This data type represents the properties describing the TAI of gNB, which is used to uniquely identify a Tracking Area.

#### 4.3.49.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNId | M | T | T | F | T |
| nRTAC | M | T | T | F | T |

#### 4.3.49.3	Attribute constraints

None.

#### 4.3.49.4	Notifications

The clause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.50	RimRSGlobal

#### 4.3.50.1	Definition

This IOC is used to represent global/common Remote Interference Management (RIM) Reference Signal (RS) resource allocated for the whole network. Resource for RIM-RS transmission is defined by Sequence domain resource, Time domain resource and Frequency resource. The configure parameters of the RIM RS resource are applied to all Sets of RIM RS Resource across gNBs/cells in the network.

#### 4.3.50.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| frequencyDomainPara | M | T | T | F | T |
| sequenceDomainPara | M | T | T | F | T |
| timeDomainPara | M | T | T | F | T |

#### 4.3.50.3	Attribute constraints

None.

#### 4.3.50.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.51	FrequencyDomainPara <<dataType>>

#### 4.3.51.1	Definition

This  data type defines configuration parameters of frequency domain resource to support RIM RS.

#### 4.3.51.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| rimRSSubcarrierSpacing | M | T | T | F | T |
| rIMRSBandwidth | M | T | T | F | T |
| nrofGlobalRIMRSFrequencyCandidates | M | T | T | F | T |
| rimRSCommonCarrierReferencePoint | M | T | T | F | T |
| rimRSStartingFrequencyOffsetIdList | M | T | T | F | T |

#### 4.3.51.3	Attribute constraints

None.

#### 4.3.51.4	Notifications

The clause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.52	SequenceDomainPara  <<dataType>>

#### 4.3.52.1	Definition

This  data type defines configuration parameters of sequence domain resource to support RIM RS.

#### 4.3.52.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nrofRIMRSSequenceCandidatesofRS1 | M | T | T | F | T |
| rimRSScrambleIdListofRS1 | M | T | T | F | T |
| nrofRIMRSSequenceCandidatesofRS2 | O | T | T | F | T |
| rimRSScrambleIdListofRS2 | O | T | T | F | T |
| enableEnoughNotEnoughIndication | M | T | T | F | T |
| RIMRSScrambleTimerMultiplier | M | T | T | F | T |
| RIMRSScrambleTimerOffset | M | T | T | F | T |

#### 4.3.52.3	Attribute constraints

None.

#### 4.3.52.4	Notifications

The clause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.53	TimeDomainPara  <<dataType>>

#### 4.3.53.1	Definition

This  data type defines configuration parameters of time domain resource to support RIM RS.

#### 4.3.53.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dlULSwitchingPeriod1 | M | T | T | F | T |
| symbolOffsetOfReferencePoint1 | M | T | T | F | T |
| dlULSwitchingPeriod2 | O | T | T | F | T |
| symbolOffsetOfReferencePoint2 | O | T | T | F | T |
| totalnrofSetIdofRS1 | M | T | T | F | T |
| totalnrofSetIdofRS2 | O | T | T | F | T |
| nrofConsecutiveRIMRS1 | M | T | T | F | T |
| nrofConsecutiveRIMRS2 | O | T | T | F | T |
| consecutiveRIMRS1List | M | T | T | F | T |
| consecutiveRIMRS2List | M | T | T | F | T |
| enablenearfarIndicationRS1 | O | T | T | F | T |
| enablenearfarIndicationRS2 | O | T | T | F | T |

#### 4.3.53.3	Attribute constraints

None.

#### 4.3.53.4	Notifications

The clause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.54	RimRSReportConf <<dataType>>

#### 4.3.54.1	Definition

This  data type defines RIM-RS reporting configuration.

#### 4.3.54.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| reportIndicator | M | T | T | F | T |
| reportInterval | M | T | T | F | T |
| nrofRIMRSReportInfo | M | T | T | F | T |
| maxPropagationDelay | O | T | T | F | T |
| RimRSReportInfoList | M | T | T | F | T |

#### 4.3.54.3	Attribute constraints

None.

#### 4.3.54.4	Notifications

The clause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.55	RimRSReportInfo <<dataType>>

#### 4.3.55.1	Definition

This  data type defines necessary reporting information derived from the detected RIM-RS, including

1)	The detected set ID;

2)	Propagation delay in number of OFDM symbols

3)	Functionality of the RS (RS-1 or RS-2, Enough or Not enough mitigation for RS-1).

#### 4.3.55.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| detectedSetID | M | T | T | F | T |
| propagationDelay | O | T | T | F | T |
| functionalityOfRIMRS | M | T | T | F | T |

#### 4.3.55.3	Attribute constraints

None.

#### 4.3.55.4	Notifications

The clause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.56	RimRSSet

#### 4.3.56.1	Definition

This IOC is used to represent aggressor or victim Set organized by OAM. The RIM RS Resource is assigned to each Set, which is identified by triple indices set of  <Time domain index, Frequency domain index, and  Sequence index >. The triple indices set can be derived by setId attribute (See clause 7.4.1.6 in TS 38.211 [32]).

#### 4.3.56.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| setId | M | T | T | F | T |
| setType | M | T | T | F | T |
| rimRSMonitoringStartTime | O | T | T | F | T |
| rimRSMonitoringStopTime | O | T | T | F | T |
| rimRSMonitoringWindowDuration | O | T | T | F | T |
| rimRSMonitoringWindowStartingOffset | O | T | T | F | T |
| rimRSMonitoringWindowPeriodicity | O | T | T | F | T |
| rimRSMonitoringOccasionInterval | O | T | T | F | T |
| rimRSMonitoringOccasionStartingOffset | O | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| nRCellDURef | M | T | F | F | T |

#### 4.3.56.3	Attribute constraints

None.

#### 4.3.56.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.57	DANRManagementFunction

#### 4.3.57.1	Definition

This IOC contains attributes to support the D-SON function of ANR Management (See clause 6.4.1.3 in TS 28.313 [57]).

#### 4.3.57.2	Attributes

The DANRManagementFunction IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| intrasystemANRManagementSwitch | M | T | T | F | T |
| intersystemANRManagementSwitch | M | T | T | F | T |

#### 4.3.57.3	Attribute constraints

None.

#### 4.3.57.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.58	DESManagementFunction

#### 4.3.58.1	Definition

This IOC represents the management capabilities of Distributed SON or Domain-Centralized SON Energy Saving (ES) functions. This is provided for Energy Saving purposes.

NOTE: in the case where multiple DESManagement MOIs exist at different levels of the containment tree, the DESManagement MOI at the lower level overrides the DESManagement MOIs at higher level(s) of the same containment tree.

#### 4.3.58.2	Attributes

The DESManagementFunction IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| desSwitch | M | T | T | F | T |
| intraRatEsActivationOriginalCellLoadParameters | CM | T | T | F | T |
| intraRatEsActivationCandidateCellsLoadParameters | CM | T | T | F | T |
| intraRatEsDeactivationCandidateCellsLoadParameters | CM | T | T | F | T |
| esNotAllowedTimePeriod | O | T | T | F | T |
| interRatEsActivationOriginalCellParameters | CM | T | T | F | T |
| interRatEsActivationCandidateCellParameters | CM | T | T | F | T |
| interRatEsDeactivationCandidateCellParameters | CM | T | T | F | T |
| energySavingState | M | T | F | F | T |
| isProbingCapable | O | T | F | F | T |

#### 4.3.58.3	Attribute constraints

| Name | Definition |
| --- | --- |
| intraRatEsActivationOriginalCellLoadParameters Support Qualifier | The condition is "Intra-RAT ESM is supported AND the cell acts as an original cell". |
| intraRatEsActivationCandidateCellsLoadParameters Support Qualifier | The condition is "Intra-RAT ESM is supported AND the cell acts as a candidate cell". |
| intraRatEsDeactivationCandidateCellsLoadParameters Support Qualifier | The condition is "Intra-RAT ESM is supported AND the cell acts as a candidate cell". |
| interRatEsActivationOriginalCellParameters CM Support Qualifier | The condition is "The cell acts as an original cell" AND inter-RAT ESM is supported. |
| interRatEsActivationCandidateCellParameters CM Support Qualifier | The condition is "The cell acts as a candidate cell" AND inter-RAT ESM is supported. |
| interRatEsDeactivationCandidateCellParameters CM Support Qualifier | The condition is "The cell acts as a candidate cell" AND inter-RAT ESM is supported. |

#### 4.3.58.4	Notification

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.59	DRACHOptimizationFunction

#### 4.3.59.1	Definition

This IOC contains attributes to support the D-SON function of RACH optimization (See clause 7.1.1 in TS 28.313 [57]).

NOTE: in the case where multiple DRACHOptimization MOIs exist at different levels of the containment tree, the DRACHOptimization MOI at the lower level overrides the DRACHOptimization MOIs at higher level(s) of the same containment tree.

#### 4.3.59.2	Attributes

The DRACHOptimizationFunction IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| ueAccProbilityDistPerSSB | M | T | T | F | T |
| ueAccDelayProbilityDistPerSSB | M | T | T | F | T |
| drachOptimizationControl | M | T | T | F | T |

#### 4.3.59.3	Attribute constraints

None.

#### 4.3.59.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.60	DMROFunction

#### 4.3.60.1	Definition

This IOC contains attributes to support the D-SON function of MRO (See clause 7.1.2 in TS 28.313 [57]).

NOTE: in the case where multiple DMRO MOIs exist at different levels of the containment tree, the DMRO MOI at the lower level overrides the DMRO MOIs at higher level(s) of the same containment tree.

#### 4.3.60.2	Attributes

The DMROFunction IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dmroControl | M | T | T | F | T |
| maximumDeviationHoTrigger | M | T | T | F | T |
| minimumTimeBetweenHoTriggerChange | M | T | T | F | T |
| tstoreUEcntxt | M | T | T | F | T |

#### 4.3.60.3	Attribute constraints

None.

#### 4.3.60.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions

### 4.3.61	DPCIConfigurationFunction

#### 4.3.61.1	Definition

This IOC contains attributes to support the Distributed SON or Domain-Centralized SON function of PCI configuration (See clause 7.1.3 in TS 28.313 [57]).

NOTE: in the case where multiple DPCIConfiguration MOIs exist at different levels of the containment tree, the DPCIConfiguration MOI at the lower level overrides the DPCIConfiguration MOIs at higher level(s) of the same containment tree.

#### 4.3.61.2	Attributes

The DPCIConfigControlFunction IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dPciConfigurationControl | M | T | T | F | T |
| nRPciList | M | T | T | F | T |

#### 4.3.61.3	Attribute constraints

None.

#### 4.3.61.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.62	CPCIConfigurationFunction

#### 4.3.62.1	Definition

This IOC contains attributes to support the Cross Domain-Centralized SON function of PCI configuration (See clause 7.1.3 in TS 28.313 [57]).

NOTE: in the case where multiple CPCIConfiguration MOIs exist at different levels of the containment tree, the CPCIConfiguration MOI at the lower level overrides the CPCIConfiguration MOIs at higher level(s) of the same containment tree.

#### 4.3.62.2	Attributes

The CPCIConfigurationFunction IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cPciConfigurationControl | M | T | T | F | T |
| cSonPciList | M | T | T | F | T |

#### 4.3.62.3	Attribute constraints

None.

#### 4.3.62.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions

### 4.3.63	CESManagementFunction

#### 4.3.63.1	Definition

This IOC represents the management capabilities of Cross Domain-Centralized SON Energy Saving (ES) functions. This is provided for Energy Saving purposes.

NOTE: in the case where multiple CESManagement MOIs exist at different levels of the containment tree, the CESManagement MOI at the lower level overrides the ESManagement MOIs at higher level(s) of the same containment tree.

#### 4.3.63.2	Attributes

The CESManagementFunction IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cesSwitch | M | T | T | F | T |
| energySavingControl | M | T | T | F | T |
| energySavingState | M | T | T | F | T |

#### 4.3.63.3	Attribute constraints

None.

#### 4.3.63.4	Notification

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.64	AddressWithVlan <<dataType>>

#### 4.3.64.1	Definition

This data type represents the address including IP address and VLAN Id (e.g. localAddress of EP_NgC) used for initialization of the underlying transport.

#### 4.3.64.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| iPAddress | O | T | T | F | T |
| vLANId | O | T | T | F | T |

#### 4.3.64.3	Attribute constraints

None

#### 4.3.64.4	Notifications

The clause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.65	TceIDMappingInfo  <<dataType>>

#### 4.3.65.1	Definition

This data type represents the properties describing the mapping relationship between TCE ID, PLMN where TCE resides and IP address of TCE.

#### 4.3.65.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| tceIPAddress | M | T | T | F | T |
| tceID | M | T | T | F | T |
| pLMNTarget | M | T | T | F | T |

#### 4.3.65.3	Attribute constraints

None.

#### 4.3.65.4	Notifications

The clause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

## 4.4	Attribute definitions

### 4.4.1	Attribute properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| administrativeState | It indicates the administrative state of the NRCellDU. It describes the permission to use or prohibition against using the cell, imposed through the OAM services. allowedValues: LOCKED, SHUTTING DOWN, UNLOCKED. The meaning of these values is as defined in ITU-T Recommendation X.731 [18]. See Annex A for Relation between the "Pre-operation state of the gNB-DU Cell" and administrative state relevant in case of 2-split and 3-split deployment scenarios. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: LOCKED isNullable: False |
| operationalState | It indicates the operational state of the NRCellDU instance. It describes whether the resource is installed and partially or fully operable (Enabled) or the resource is not installed or not operable (Disabled). allowedValues: ENABLED, DISABLED. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cellState | It indicates the usage state of the NRCellDU instance. It describes whether the cell is not currently in use (Idle), or currently in use but not configured to carry traffic (Inactive) or is currently in use and is configured to carry traffic (Active). The Inactive and Active definitions are in accordance with TS 38.401 [4]: "Inactive: the cell is known by both the gNB-DU and the gNB-CU. The cell shall not serve UEs; Active: the cell is known by both the gNB-DU and the gNB-CU. The cell should be able to serve UEs." "allowedValues: IDLE, INACTIVE, ACTIVE. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| arfcnDL | NR Absolute Radio Frequency Channel Number (NR-ARFCN) for downlink allowedValues: See TS 38.104 [12] clause 5.4.2. Note that allowed values of NR-ARFCN are specified for each band in clause 5.4.2.3. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| arfcnUL | NR Absolute Radio Frequency Channel Number (NR-ARFCN) for uplink allowedValues: See TS 38.104 [12] clause 5.4.2. Note that allowed values of NR-ARFCN are specified for each band in clause 5.4.2.3. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| arfcnSUL | NR Absolute Radio Frequency Channel Number (NR-ARFCN) for supplementary uplink allowedValues: See TS 38.104 [12] clause 5.4.2. Note that allowed values of NR-ARFCN are specified for each band in clause 5.4.2.3. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| beamAzimuth | The azimuth of a beam transmission, which means the horizontal beamforming pointing angle (beam peak direction) in the (Phi) φ-axis in 1/10th degree resolution. See clauses 3.2 in TS 38.104 [12] and 7.3 in TS 38.901 [53] as well as TS 28.662 [11]. The pointing angle is the direction equal to the geometric centre of the half-power contour of the beam relative to the reference plane. Zero degree implies explicit antenna bearing (boresight). Positive angle implies clockwise from the antenna bearing. allowedValues: [-1800 ..1800] 0.1 degree | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Null isNullable: True |
| beamHorizWidth | The Horizontal beamWidth of a beam transmission, which means the horizontal beamforming half-power (3dB down) beamwidth in the (Phi) φ-axis in 1/10th degree resolution. See clauses 3.2 in TS 38.104 [12] and 7.3 in TS 38.901 [53]. allowedValues: [0..3599] 0.1 degree | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Null isNullable: True |
| beamIndex | Index of the beam. For example, please see clause 6.6.2 of TS 38.331 [54] where the ssb-Index in the rsIndexResults element of MeasResultNR is defined. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Null isNullable: True |
| beamTilt | The tilt of a beam transmission, which means the vertical beamforming pointing angle (beam peak direction) in the (Theta) θ-axis in 1/10th degree resolution. See clauses 3.2 in TS 38.104 [12] and 7.3 in TS 38.901 [53] as well as TS 28.662 [11]. The pointing angle is the direction equal to the geometric centre of the half-power contour of the beam relative to the reference plane. Positive value implies downtilt. allowedValues: [-900..900] 0.1 degree | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Null isNullable: True |
| beamType | The type of the beam. allowedValues: "SSB-BEAM" | type: string multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: Null isNullable: True |
| beamVertWidth | The Vertical beamWidth of a beam transmission, which means the vertical beamforming half-power (3dB down) beamwidth in the (Theta) θ-axis in 1/10th degree resolution. See clauses 3.2 in TS 38.104 [12] and 7.3 in TS 38.901 [53]. allowedValues: [0...1800] 0.1 degree | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Null isNullable: True |
| bSChannelBwDL | BS Channel BW in MHz. for downlink allowedValues: See BS Channel BW in TS 38.104 [12], clause 5.3.​ | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| bSChannelBwUL | BS Channel BW in MHz.for uplink allowedValues: See BS Channel BW in TS 38.104 [12], clause 5.3.​ | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| bSChannelBwSUL | BS Channel BW in MHz.for supplementary uplink allowedValues: See BS Channel BW in TS 38.104 [12], clause 5.3.​ | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| configuredMaxTxPower | This is the maximum transmission power in milliwatts (mW) at the antenna port for all downlink channels, used simultaneously in a cell, added together. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| configuredMaxTxEIRP | This is the maximum emitted isotroptic radiated power (EIRP) in dBm for all downlink channels, used simultaneously in a cell, added together [12]. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| coverageShape | Identifies the sector carrier coverage shape described by the envelope of the contained SSB beams. The coverage shape is implementation dependent. allowedValues: 0 : 65535 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| digitalTilt | Digitally-controlled tilt through beamforming. It represents the vertical pointing direction of the antenna relative to the antenna bore sight, representing the total non-mechanical vertical tilt of the selected coverageShape. Positive value gives downwards tilt and negative value gives upwards tilt. allowedValues: [-900..900] 0.1 degree | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| digitalAzimuth | Digitally-controlled azimuth through beamforming. It represents the horizontal pointing direction of the antenna relative to the antenna bore sight, representing the total non-mechanical horizontal pan of the selected coverageShape. Positive value gives azimuth to the right and negative value gives an azimuth to the left. allowedValues: [-1800 ..1800] 0.1 degree | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cyclicPrefix | Cyclic prefix as defined in TS 38.211 [32], clause 4.2. allowedValues: NORMAL, EXTENDED. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| localAddress | This parameter specifies the localAddress used for initialization of the underlying transport. The AddressWithVlan <dataType> is defined in clause 4.3.64. | type: AddressWithVlan multiplicity: 1 isOrdered: False isUnique: N/A defaultValue: None isNullable: False |
| AddressWithVlan.iPaddress | This parameter specifies the IP address used for initialization of the underlying transport. IP address can be an IPv4 address (See RFC 791 [37]) or an IPv6 address (See RFC 2373 [38]). | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| AddressWithVlan. vlanId | This parameter specifies the local VLAN Id (See IEEE 802.1Q [39]) used for initialization of the underlying transport. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| remoteAddress | Remote address including IP address used for initialization of the underlying transport. IP address can be an IPv4 address (See RFC 791 [37]) or an IPv6 address (See RFC 2373 [38]). | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gNBId | It identifies a gNB within a PLMN. The gNB ID is part of the NR Cell Identifier (NCI) of the gNB cells. See "gNB Identifier (gNB ID)" of clause 8.2 of TS 38.300 [3]. See "Global gNB ID" in clause 9.3.1.6 of TS 38.413 [5]. allowedValues: 0..4294967295 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gNBIdLength | This indicates the number of bits for encoding the gNB ID. See "Global gNB ID" in clause 9.3.1.6 of TS 38.413 [5]. allowedValues: 22 .. 32. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gNBDUId | It uniquely identifies the DU at least within a gNB-CU. See 'gNB-DU ID' in clause 9.3.1.9 of TS 38.473 [8]. allowedValues: 0..236-1 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gNBCUUPId | It uniquely identifies the gNB-CU-UP at least within a gNB-CU-CP. See 'gNB-CU-UP ID' in clause 9.3.1.15 of TS 38.463 [48]. allowedValues: 0..236-1 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gNBCUName | It identifies the Central Entity of a NR node, see clause 9.2.1.4 of TS 38.473 [8]. allowedValues: Not applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gNBDUName | It identifies the Distributed Entity of a NR node, see clause 9.2.1.5 of TS 38.473 [8]. allowedValues: Not applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cellLocalId | It identifies a NR cell of a gNB. It, together with the gNB Identifier (using gNBId of the parent GNBCUCPFunction or GNBDUFunction or ExternalCUCPFunction), identifies a NR cell within a PLMN. This is the NR Cell Identity (NCI). See clause 8.2 of TS 38.300 [3]. The NCI can be constructed by encoding the gNB Identifier using gNBId (of the parent GNBCUCPFunction or GNBDUFunction or ExternalCUCPFunction) and cellLocalId where the gNB Identifier field is of length specified by gNBIdLength (of the parent GNBCUCPFunction or GNBDUFunction or ExternalCUCPFunction). See "Global gNB ID" in clause 9.3.1.6 of TS 38.413 [5]. The NR Cell Global identifier (NCGI) is constructed from the PLMN identity the cell belongs to and the NR Cell Identifier (NCI) of the cell. See relation between NCI and NCGI clause 8.2 of TS 38.300 [3]. allowedValues: Not applicable | type: Integer multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| nRPCI | This holds the Physical Cell Identity (PCI) of the NR cell. allowedValues: See TS 36.211 clause 6.11 for legal values of pci. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nRTAC | This holds the identity of the common Tracking Area Code for the PLMNs. allowedValues: a) It is the TAC or Extended-TAC. b) A cell can only broadcast one TAC or Extended-TAC. See TS 36.300, clause 10.1.7 (PLMNID and TAC relation). c) TAC is defined in clause 19.4.2.3 of TS 23.003 [13] and Extended-TAC is defined in clause 9.3.1.29 of TS 38.473 [8]. d) For a 5G SA (Stand Alone), it has a non-null value. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: NULL isNullable: True |
| GNBCUCPFunction.pLMNId | It specifies the PLMN identifier to be used as part of the global RAN node identity. allowedValues: Not applicable. | Type: PLMNId multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| GNBCUUPFunction.pLMNIdList | This is a list of PLMN identifiers. It defines from which set of PLMNs an UE must have as its serving PLMN to be allowed to use the GNB-CU-UP. allowedValues: Not applicable. | type: PLMNId multiplicity: 1..12 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| NRCellCU.pLMNInfoList | It defines which PLMNs that can be served by the NR cell,and which S-NSSAIs can be supported by the NR cell for corresponding PLMN in case of network slicing feature is supported allowedValues: Not applicable. | type: PLMNInfo multiplicity: 1..* isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| NRCellDU.pLMNInfoList | It defines which PLMNs that can be served by the NR cell, and which S-NSSAs can be supported by the NR cell for corresponding PLMN in case of network slicing feature is supported. The pLMNId of the first entry of the list is the PLMNId used to construct the nCGI for the NR cell. allowedValues: Not applicable. | type: PLMNInfo multiplicity: 1..* isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| ExternalNRCellCU.pLMNIdList | It defines which PLMNs that are assumed to be served by the NR Cell in another gNB-CU-CP. This list is either updated by the managed element itself (e.g. due to ANR, signalling over Xn etc) or by consumer over the standard interface. allowedValues: Not applicable. | Type: PLMNId multiplicity: 1..12 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| rRMPolicyMemberList | It represents the list of RRMPolicyMember (s) that the managed object is supporting. A RRMPolicyMember <<dataType>> include the PLMNId <<dataType>> and S-NSSAI <<dataType>>. allowedValues: N/A | type: RRMPolicyMember multiplicity: 1..* isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| resourceType | The resource type of interest for an RRM Policy. allowedValues: PRB (for NRCellDU, GNBDUFunction) RRC connected users (for NRCellCU, GNBCUCPFunction) DRB (for GNBCUUPFunction) See NOTE 2and NOTE 4 | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| sNSSAIList | It represents the list of S-NSSAI the managed object is supporting. The S-NSSAI is defined in TS 23.003 [13]. allowedValues: See TS 23.003 [13] | type: S-NSSAI multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| sST | This attribute specifies the Slice/Service type (SST) of the network slice. See clause 5.15.2 of TS 23.501 [2]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| sD | This attribute specifies the Slice Differentiator (SD), which is optional information that complements the slice/service type(s) to differentiate amongst multiple Network Slices. See clause 5.15.2 of TS 23.501 [2]. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| rRMPolicyMaxRatio | This attribute specifies the maximum percentage of radio resources that can be used by the associated rRMPolicyMemberList. The maximum percentage of radio resources include at least one of the shared resources, prioritized resources and dedicated resources. The sum of the 'rRMPolicyMaxRatio' values assigned to all RRMPolicyRatio(s) name-contained by same MangedEntity can be greater than 100. Default value: 100 allowedValues: 0 : 100 | type: Integer multiplicity: isOrdered: N/A isUnique: N/A defaultValue: True allowedValues: N/A isNullable: False |
| rRMPolicyMinRatio | This attribute specifies the minimum percentage of radio resources that can be used by the associated rRMPolicyMemberList. The minimum percentage of radio resources including at least one of prioritized resources and dedicated resources. The sum of the 'rRMPolicyMinRatio' values assigned to all RRMPolicyRatio(s) name-contained by same MangedEntity shall be less or equal 100. Default value: 0 allowedValues: 0 : 100 NOTE: Void. | type: Integer multiplicity: isOrdered: N/A isUnique: N/A defaultValue: True allowedValues: N/A isNullable: False |
| rRMPolicyDedicatedRatio | This attribute specifies the percentage of radio resource that dedicatedly used by the associated rRMPolicyMemberList. The sum of the 'rRMPolicyDedicatedRatio' values assigned to all RRMPolicyRatio(s) name-contained by same MangedEntity shall be less or equal 100. Default value: 0 allowedValues:0 : 100 | type: Integer multiplicity: isOrdered: N/A isUnique: N/A defaultValue: TRUE allowedValues: N/A isNullable: False |
| subCarrierSpacing | Subcarrier spacing configuration for a BWP. See clause 5 in TS 38.104 [12]. AllowedValues: [15, 30, 60, 120] depending on the frequency range FR1 or FR2. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| txDirection | Indicates if the transmission direction is downlink (DL), uplink (UL) or both downlink and uplink (DL and UL). allowedValues: DL, UL, DL and UL | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| bwpContext | It identifies whether the object is used for downlink, uplink or supplementary uplink. allowedValues: DL, UL, SUL | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| isInitialBwp | It identifies whether the object is used for initial or other BWP. allowedValues: INITIAL, OTHER | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| startRB | Offset in common resource blocks to common resource block 0 for the applicable subcarrier spacing for a BWP. This corresponds to N_BWP_start, see clause 4.4.5 in TS 38.211 [32]. allowedValues: 0 to N_grid_size – 1, where N_grid_size equals the number of resource blocks for the BS channel bandwidth, given the subcarrier spacing of the BWP. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| numberOfRBs | Number of physical resource blocks for a BWP. This corresponds to N_BWP_size, see clause 4.4.5 in TS 38.211 [32]. allowedValues: 1 to N_grid_size – startRB of the BWP. Se startRB for definition of N_grid_size. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nRTCI | This is the Target NR Cell Identifier. It consists of NR Cell Identifier (NCI) and Physical Cell Identifier of the target NR cell (nRPCI). The NRRelation.nRTCI identifies the target cell from the perspective of the NRCell, the name-containing instance of the subject NRCellCU instance. allowedValues: Not applicable. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| adjacentCellRef | This attribute contains the DN of an adjacentNRCell (NRCellCU or ExternalNRCellCU) allowedValues: Not applicable. | type: DN multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| ssbFrequency | Indicates cell defining SSB frequency domain position Frequency of the cell defining SSB transmission. The frequency provided in this attribute identifies the position of resource element RE=#0 (subcarrier #0) of resource block RB#10 of the SS block. The frequency must be positioned on the NR global frequency raster, as defined in TS 38.101-1 [42] clause 5.4.2. and within bSChannelBwDL. allowedValues: 0..3279165 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nRFrequencyRef | This attribute contains the DN of the referenced NRFrequency. allowedValues: Not applicable. | type: DN multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| nRSectorCarrierRef | This attribute contains the DN of the referenced NRSectorCarrier. allowedValues: Not applicable. | type: DN multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| bWPRef | This attribute contains the DN of the referenced BWP. allowedValues: Not applicable. | type: DN multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| sectorEquipmentFunctionRef | This attribute contains the DN of the referenced NSectorEquipmentFunction. allowedValues: Not applicable. | type: DN multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| offsetMO | It is a list of offset values applicable to all measured cells with reference signal(s) indicated in this MeasObjectNR. See offsetMO of clause 5.5.4 of TS 38.331 [54]. allowedValues: Not applicable. | type: QOffsetRangeList multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: N/A isNullable: False |
| cellIndividualOffset | It is a list of offset values for the neighbour cell. Used when UE is in connected mode. The unit is 1dB. It is defined for rsrpOffsetSSB, rsrqOffsetSSB, sinrOffsetSSB, rsrpOffsetCSI-RS, rsrqOffsetCSI-RS and sinrOffsetCSI-RS. See TS 38.331 [54]. allowedValues: Not applicable. | type: Integer multiplicity: 6 isOrdered: True isUnique: N/A defaultValue: 0 isNullable: False |
| blackListEntry | It specifies a list of PCI (physical cell identity) that are blacklisted in EUTRAN measurements as described in TS 38.331 [54]. allowedValues: { 0…1007 } | type: Integer multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| blackListEntryIdleMode | It specifies a list of PCI (physical cell identity) that are blacklisted in SIB4 and SIB5. allowedValues: { 0…1007 } | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cellReselectionPriority | It is the absolute priority of the carrier frequency used by the cell reselection procedure. See CellReselectionPriority IE in TS 38.331 [54]. It corresponds to the parameter priority in TS 38.304 [49]. Value 0 means lowest priority. The UE behaviour when no value is entered is specified in clause 5.2.4.1 of TS 38.304 [49]. The value must not already used by other RAT, i.e. equal priorities between RATs are not supported. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 0None isNullable: False |
| cellReselectionSubPriority | It indicates a fractional value to be added to the value of cellReselectionPriority to obtain the absolute priority of the concerned carrier frequency for E-UTRA and NR. See CellReselectionSubPriority IE in TS 38.331 [54]. allowedValues: { 0.2, 0.4, 0.6, 0.8 }. | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| pMax | It calculates the parameter Pcompensation (defined in TS 38.304 [49]), at cell reselection to an Cell. Its unit is 1 dBm. It corresponds to parameter PEMAX in TS 38.101-1 [42]. allowedValues: { -30..33 }. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| qOffsetFreq | It is the frequency specific offset applied when evaluating candidates for cell reselection. See TS 38.331 [49]. Its unit is 1 dB. allowedValues: { -24, -22, -20, -18, -16, -14, -12, -10, -8, -6, -5, -4, -3, -2, -1, 0, 1, 2, 3, 4, 5, 6, 8, 10, 12, 14, 16, 20, 22, 24 } | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 0 isNullable: False |
| qOffsetRangeList | It is used to indicate a cell, beam or measurement object specific offset to be applied when evaluating candidates for cell re-selection or when evaluating triggering conditions for measurement reporting. The value in dB. Value dB-24 corresponds to -24 dB, dB-22 corresponds to -22 dB and so on. This is a list of enum values representing, in sequence: rsrpOffsetSSB, rsrqOffsetSSB, sinrOffsetSSB, rsrpOffsetCSI-RS, srqOffsetCSI-RS, sinrOffsetCSI-RS. See Q-OffsetRangeList in clause of clause 6.3.1 of TS 38.331 [54]. allowedValues: { -24, -22, -20, -18, -16, -14, -12, -10, -8, -6, -5, -4, -3, -2, -1, 0, 1, 2, 3, 4, 5, 6, 8, 10, 12, 14, 16, 18, 20, 22, 24 } | type: ENUM multiplicity: 6 isOrdered: True isUnique: N/A defaultValue: 0 isNullable: False |
| qQualMin | It indicates the minimum required quality level in the cell (dB). See qQualMin in TS 38.304 [49]. Unit is 1 dB. Value 0 means that it is not sent and UE applies in such case the (default) value of negative infinity for Qqualmin. Sent in SIB3 or SIB5. allowedValues: { -34..-3, 0 } | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| qRxLevMin | It indicates the required minimum received Reference Symbol Received Power (RSRP) level in the (E-UTRA) frequency for cell reselection. It corresponds to Qrxlevmin defined in TS 38.304 [49]. It is broadcast in SIB3 or SIB5, depending on whether the related frequency is intra- or inter-frequency. Its unit is 1 dBm and resolution is 2. allowedValues: { -140..-44 }. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| threshXHighP | This specifies the Srxlev threshold (in dB) used by the UE when reselecting towards a higher priority RAT/ frequency than the current serving frequency. Each frequency of NR and E-UTRAN might have a specific threshold. It corresponds to the ThreshX, HighP in TS 38.304 [49]. Its unit is 1 dB and resolution is 2. allowedValues: { 0..62 } | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| threshXHighQ | This specifies the Squal threshold (in dB) used by the UE when reselecting towards a higher priority RAT/ frequency than the current serving frequency. Each frequency of NR and E-UTRAN might have a specific threshold. It corresponds to the ThreshX, HighQ in TS 38.304 [49]. Its unit is 1 dB. allowedValues: { 0..31 } | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| threshXLowP | This specifies the Srxlev threshold (in dB) used by the UE when reselecting towards a lower priority RAT/ frequency than the current serving frequency. Each frequency of NR might have a specific threshold. It corresponds to ThreshX,LowP in TS 38.304 [49]. Its unit is 1 dB. Its resolution is 2. allowedValues: { 0..62 } | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| threshXLowQ | This specifies the Squal threshold (in dB) used by the UE when reselecting towards a lower priority RAT/ frequency than the current serving frequency. Each frequency of NR might have a specific threshold. It corresponds to ThreshX,Low in TS 38.304 [49]. Its unit is 1 dB. allowedValues: {0..31}. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| tReselectionNr | It is the cell reselection timer and corresponds to parameter TreselectionRAT for NR defined in 38.331 [54]. Its unit is in seconds. allowedValues: {0..7}. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| tReselectionNRSfHigh | The attribute t-ReselectionNr (a parameter TreselectionNR in TS 38.304 [49]) is multiplied with this factor if the UE is in high mobility state. It corresponds to the parameter Speed dependent ScalingFactor for TreselectionNr for medium high state in TS 38.304 [49]. The unit is one %. Value mapping: 25 = 0.25 50 = 0.5 75 = 0.75 100 = 1.0 allowedValues: {25, 50, 75, 100}. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| tReselectionNRSfMedium | The attribute t-ReselectionNR (a parameter "TreselectionNR in TS 38.304 [49]") is multiplied with this factor if the UE is in medium mobility state. It corresponds to the parameter Speed dependent ScalingFactor for TreselectionNr for medium mobility state in TS 38.304 [49]. Its unit is one %. Value mapping: 25 = 0.25 50 = 0.5 75 = 0.75 100 = 1.0 allowedValues: {25, 50, 75, 100}. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| absoluteFrequencySSB | The absolute frequency applicable for a downlink NR carrier frequency associated with the SSB. allowedValues: {0.. 3279165}. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| sSBSubCarrierSpacing | This SSB is used for for synchronization. See clause 5 in TS 38.104 [12]. Its units are in kHz. allowedValues: {15, 30, 120, 240}. Note that the allowed values of SSB used for representing data, by e.g. a BWP, are: 15, 30, 60 and 120 in units of kHz. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| multiFrequencyBandListNR | It is a list of additional frequency bands the frequency belongs to. The list is automatically set by the gNB. allowedValues: {1..256 } | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ssbPeriodicity | Indicates cell defined SSB periodicity in number of subframes (ms). The SSB periodicity in msec is used for the rate matching purpose. allowedValues: 5, 10, 20, 40, 80, 160. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ssbOffset | Indicates cell defining SSB time domain position. Defined as the offset of the measurement window, in number of subframes (ms), in which to receive SS/PBCH blocks, where allowed values depend on the ssbPeriodicity. allowedValues: ssbPeriodicity5 ms 0..4, ssbPeriodicity10 ms 0..9, ssbPeriodicity20 ms 0..19, ssbPeriodicity40 ms 0..39, ssbPeriodicity80 ms 0..79, ssbPeriodicity160 ms 0..159. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ssbDuration | Duration of the measurement window in which to receive SS/PBCH blocks. It is given in number of subframes (ms) (see 38.213 [41], clause 4.1. allowedValues: 1, 2, 3, 4, 5. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rimRSMonitoringStartTime | This field configures the UTC time when the gNB attempts to start RIM-RS monitoring. allowedValues: containing the information same with xsd: dateTime. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rimRSMonitoringStopTime | This field configures the UTC time when the gNB stops RIM-RS monitoring. allowedValues: containing the information same with xsd: dateTime. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mappingSetIDBackhaulAddressList | The attribute specifies a list of mappingSetIDBackhaulAddress which is defined as a datatype (see clause 4.3.47). Which is used to retrieve the backhaul address of the victim set. allowedValues: Not applicable | type: MappingSetIDBackhaulAddress multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| backhaulAddress | The attribute specifies backhaulAddress which is defined as a datatype (see clause 4.3.48). allowedValues: Not applicable | type: BackhaulAddress multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| setID | This specifies the set ID of a victim Set (RIM-RS1 Set) or aggressor Set (RIM-RS2 set). (See clause 7.4.1.6 in TS 38.211 [32]). allowedValues: The bit length of the set ID is maximum 22bit. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| tAI | Indicates the TAI (see clause 9.3.3.11 in TS 38.413[5]), including pLMNId ID and nRTAC. allowedValues: Not applicable | type: TAI multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| isRemoveAllowed | This indicates if the subject NRCellRelation can be removed (deleted) or not. If TRUE, the subject NRCellRelation instance can be removed (deleted). If FALSE, the subject NRCellRelation instance shall not be removed (deleted) by any entity but an MnS consumer. allowedValues: TRUE,FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| isHOAllowed | This indicates if HO is allowed or prohibited. If TRUE, handover is allowed from source cell to target cell. The source cell is identified by the name-containing NRCellCU of the NRCellRelation that contains the isHOAllowed. The target cell is referenced by the NRCellRelation that contains this isHOAllowed. If FALSE, handover shall not be allowed. allowedValues: TRUE,FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| intrasystemANRManagementSwitch | This attribute determines whether the intra-system ANR function is activated or deactivated. If "TRUE", the intra-system ANR function may add or remove intra NG-RAN Neighbour Relations, i.e. add or remove NRCellRelation instances from NRCellCU of this GNBCUCPFunction. If "FALSE", the intra-system ANR Function must not add or remove Neighbour Relations, i.e. add or remove NRCellRelation instances from NRCellCU of this GNBCUCPFunction. allowedValues: TRUE,FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| intersystemANRManagementSwitch | This attribute determines whether the inter-system ANR function is activated or deactivated. If "TRUE", the inter-system ANR function may add or remove inter-system Neighbour Relations, i.e. add or remove EUtranRelation instances from NRCellCU of this GNBCUCPFunction. If "FALSE", the inter-system ANR Function must not add or remove inter-system Neighbour Relations, i.e. add or remove EUtranRelation instances from NRCellCU of this GNBCUCPFunction. allowedValues: TRUE,FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| desSwitch | This attribute determines whether the Distributed SON or Domain-Centralized SON energy saving function is enabled or disabled. allowedValues: TRUE,FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cesSwitch | This attribute determines whether the Cross Domain-Centralized SON energy saving function is enabled or disabled. allowedValues: TRUE,FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| energySavingControl | This attribute allows the Cross Domain-Centralized SON energy saving function to initiate energy saving activation or deactivation. allowedValues: toBeEnergySaving, toBeNotEnergySaving | type: enumeration multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| energySavingState | Specifies the status regarding the energy saving in the cell. If the value of energySavingControl is toBeEnergySaving, then it shall be tried to achieve the value isEnergySaving for the energySavingState. If the value of energySavingControl is toBeNotEnergySaving, then it shall be tried to achieve the value isNotEnergySaving for the energySavingState. allowedValues: isNotEnergySaving, isEnergySaving. | type: enumeration multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| intraRatEsActivationOriginalCellLoadParameters | This attributes is relevant, if the cell acts as an original cell. This attribute indicates the traffic load threshold and the time duration, which are used by distributed ES algorithms to allow a cell to enter the energySaving state. The time duration indicates how long the load needs to have been below the threshold. allowedValues: Threshold: Integer 0..100 (Percentage of PRB usage, see TS 36.314 [13]) TimeDuration: Integer (in unit of seconds) | type: data type multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| intraRatEsActivationCandidateCellsLoadParameters | This attributes is relevant, if the cell acts as a candidate cell. This attribute indicates the traffic load threshold and the time duration, which are used by distributed ES algorithms level to allow a n 'original' cell to enter the energySaving state. Threshold and duration are applied to the candidate cell(s) which will provides coverage backup of an original cell when it is in the energySaving state. The threshold applies in the same way for a candidate cell, no matter for which original cell it will provide backup coverage. The time duration indicates how long the traffic in the candidate cell needs to have been below the threshold before any original cells which will be provided backup coverage by the candidate cell enters energy saving state. allowedValues: Threshold: Integer 0..100 (Percentage of PRB usage (see TS 36.314 [13]) ) TimeDuration: Integer (in unit of seconds) | type: data type multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| intraRatEsDeactivationCandidateCellsLoadParameters | This attributes is relevant, if the cell acts as a candidate cell. This attribute indicates the traffic load threshold and the time duration which is used by distributed ES algorithms to allow a cell to leave the energySaving state. Threshold and time duration are applied to the candidate cell when it which provides coverage backup for the cell in energySaving state. The threshold applies in the same way for a candidate cell, no matter for which original cell it provides backup coverage. The time duration indicates how long the traffic in the candidate cell needs to have been above the threshold to wake up one or more original cells which have been provided backup coverage by the candidate cell. allowedValues: Threshold: Integer 0..100 (Percentage of PRB usage (see TS 36.314 [13]) ) TimeDuration: Integer (in unit of seconds) | type: data type multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| esNotAllowedTimePeriod | This attribute can be used to prevent a cell entering energySaving state. This attribute indicates a list of time periods during which inter-RAT energy saving is not allowed. Time period is valid on the specified day and time of every week. allowedValues: The legal values are as follows: startTime and endTime: All values that indicate valid UTC time. endTime should be later than startTime. periodOfDay: structure of startTime and endTime. daysOfWeekList: list of weekday. weekday: Monday, Tuesday, … Sunday. List of time periods: {{ daysOfWeek daysOfWeekList, periodOfDay dailyPeriod}} | type: data type multiplicity: 0..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| interRatEsActivationOriginalCellParameters | This attribute is relevant, if the cell acts as an original cell. This attribute indicates the traffic load threshold and the time duration, which are used by distributed inter-RAT ES algorithms to allow an original cell to enter the energySaving state. The time duration indicates how long the traffic load (both for UL and DL) needs to have been below the threshold. In case the original cell is an EUTRAN cell, the load information refers to Composite Available Capacity Group IE (see TS 36.413 [12] Annex B.1.5) and the following applies: Load = (100 - 'Capacity Value' ) * 'Cell Capacity Class Value', where 'Capacity Value' and 'Cell Capacity Class Value' are defined in TS 36.423 [7]. In case the original cell is a UTRAN cell, the load information refers to Cell Load Information Group IE (see TS 36.413 [12] Annex B.1.5) and the following applies: Load= 'Load Value' * 'Cell Capacity Class Value', where 'Load Value' and 'Cell Capacity Class Value' are defined in TS 25.413 [19]. If the 'Cell Capacity Class Value' is not known, then 'Cell Capacity Class Value' should be set to 1 when calculating the load, and the load threshold should be set in range of 0..100. allowedValues: LoadThreshold: Integer 0..10000 TimeDuration: Integer 0..900 (in unit of seconds) | type: data type multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| interRatEsActivationCandidateCellParameters | This attribute is relevant, if the cell acts as a candidate cell. This attribute indicates the traffic load threshold and the time duration, which are used by distributed inter-RAT ES algorithms to allow an original cell to enter the energySaving state. Threshold and time duration are applied to the candidate cell(s) which will provides coverage backup of an original cell when it is in the energySaving state. The time duration indicates how long the traffic load (both for UL and DL) in the candidate cell needs to have been below the threshold before any original cells which will be provided backup coverage by the candidate cell enters energySaving state. In case the candidate cell is a UTRAN or GERAN cell, the load information refers to Cell Load Information Group IE(see TS 36.413 [12] Annex B.1.5) and the following applies: Load= 'Load Value' * 'Cell Capacity Class Value', where 'Load Value' and 'Cell Capacity Class Value' are defined in TS 25.413 [19] (for UTRAN) / TS 48.008 [20] (for GERAN). If the 'Cell Capacity Class Value' is not known, then 'Cell Capacity Class Value' should be set to 1 when calculating the load, and the load threshold should be set in range of 0..100. allowedValues: LoadThreshold: Integer 0..10000 TimeDuration: Integer 0..900 (in unit of seconds) | type: data type multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| interRatEsDeactivationCandidateCellParameters | This attribute is relevant, if the cell acts as a candidate cell. This attribute indicates the traffic load threshold and the time duration which is used by distributed inter-RAT ES algorithms to allow an original cell to leave the energySaving state. Threshold and time duration are applied to the candidate cell which provides coverage backup for the cell in energySaving state. The time duration indicates how long the traffic load (either for UL or DL) in the candidate cell needs to have been above the threshold to wake up one or more original cells which have been provided backup coverage by the candidate cell. For the load see the definition of interRatEsActivationCandidateCellParameters. allowedValues: LoadThreshold: Integer 0..10000 TimeDuration: Integer 0..900 (in unit of seconds) | type: data type multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| isProbingCapable | This attribute indicates whether this cell is capable of performing the ES probing procedure. During this procedure the eNB owning the cell indicates its presence to UEs for measurement purposes, but prevents idle mode UEs from camping on the cell and prevents incoming handovers to the same cell. If this parameter is absent, then probing is not done. allowedValues: yes, no | type: enumeration multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| dmroControl | This attribute determines whether the MRO function is enabled or disabled. allowedValues: TRUE,FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cSonPciList | This holds a list of physical cell identities that can be assigned to the pci attribute by gNB. The assignment algorithm is not specified. This attribute shall be supported if and only if the C-SON PCI configuration is supported. See TS 28.313, ref [57] clause 7.1.3. allowedValues: See TS 38.211 [32] clause 7.4.2.1 for legal values of pci. The number of pci in the list is 1 to 100X. | type: Integer multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ueAccProbilityDistPerSSB | This is a list of target Access Probability (APn) for the RACH optimization function. Each instance APn of the list is the probability that the UE gets access on the RACH channel per SSB within n number of preambles sent over an unspecified sampling period. This target is suitable for RACH optimization. allowedValues: Each element of the list, APn, is a pair (a, n) where a is the targetProbability (in %) and n is the number of preambles sent. The legal values for a are 25, 50, 75, 90. The legal values for n are 1 to 200. The number of elements specified is 4. The number of elements supported is vendor specific. The choice of supported values for a and n is vendor-specific. | type: data type multiplicity: 0..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| ueAccDelayProbilityDistPerSSB | This is a list of target Access Delay probability (ADP) for the RACH optimization function. Each instance ADP of the list is the target time before the UE gets access on the RACH channel per SSB, for the P percent of the successful RACH Access attempts with lowest access delay, over an unspecified sampling period. This target is suitable for RACH optimization. allowedValues: Each element of the list, ADp, is a pair (p, d) where p is the targetProbability (in %) and d is the access delay (in milliseconds). The legal values for p are 25, 50, 75, 90. The legal values for d are 10 to 560. The number of elements specified is 4. The number of elements supported is vendor specific. The choice of supported values for a and b is vendor-specific. | type: data type multiplicity: 0..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| drachOptimizationControl | This attribute determines whether the RACH Optimization function is enabled or disabled. allowedValues: TRUE,FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nRPciList | This holds a list of physical cell identities that can be assigned to the NR cells. This attribute shall be supported if D-SON PCI configuration or domain Centralized SON PCI configuration function is supported. See clause 8.2.3, 8.3.1 in TS 28.313 [57]. allowedValues: See TS 38.211 [32] clause 7.4.2 for legal values of pci. The number of pci in the list is 0 to 1007. | type: Integer multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dPciConfigurationControl | This attribute determines whether the Distributed SON or Domain-Centralized SON PCI configuration Function is enabled or disabled. allowedValues: TRUE,FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cPciConfigurationControl | This attribute determines whether the Cross Domain-Centralized SON PCI configuration function is enabled or disabled. allowedValues: TRUE,FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| maximumDeviationHoTrigger | This parameter defines the maximum allowed absolute deviation of the Handover Trigger, from the default point of operation (see TS 38.300 [3] and TS 38.423 [58]). Editor's note: The clause references to TS 38.300 and TS 38.423 will be added, when they are available. allowedValues: -20..20 Unit: 0.5 dB | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| minimumTimeBetweenHoTriggerChange | This parameter defines the minimum allowed time interval between two Handover Trigger change performed by MRO. This is used to control the stability and convergence of the algorithm (see TS 38.300 [3]). Editor's note: The clause references to TS 38.300 will be added, when they are available. allowedValues: 0..604800 Unit: Seconds | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| tstoreUEcntxt | The timer used for detection of too early HO, too late HO and HO to wrong cell. Corresponds to Tstore_UE_cntxt timer described in TS 38.300 [3]. Editor's note: The clause references to TS 38.300 will be added, when they are available. This attribute is used for Mobility Robustness Optimization. allowedValues: 0..1023 Unit: 100 milliseconds | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| configurable5QISetRef | This is the DN of Configurable5QISet. allowedValues: DN of the Configurable5QISet MOI. | type: String multiplicity: 0..1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| dynamic5QISetRef | This is the DN of Dynamic5QISet. allowedValues: DN of the Dynamic5QISet MOI. | type: String multiplicity: 0..1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| frequencyDomainPara | This attribute defines configuration parameters of frequency domain resource to support RIM RS. allowedValues: Not applicable. | type: FrequencyDomainPara multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| sequenceDomainPara | This attribute defines configuration parameters of sequence domain resource to support RIM RS. allowedValues: Not applicable. | type: SequenceDomainPara multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| timeDomainPara | This attribute defines configuration parameters of time domain resource to support RIM RS. allowedValues: Not applicable. | type: TimeDomainPara multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rimRSSubcarrierSpacing | It is the subcarrier spacing configuration () for the RIM-RS. Subcarrier spacing (see 38.211 [32], clause 5.3.3). allowedValues: 0, 1 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rIMRSBandwidth | It is RIM RS bandwidth configuration in number of PRBs (see 38.211 [32], clause 5.3.3). For carrier bandwidth larger than 20MHz, this attributer should be 96 if subcarrier spacing is15kHz; 48 or 96 if subcarrier spacing is 30kHz; For carrier bandwidth smaller than or equal to 20MHz, this attributer should be Minimum of {96 , bandwidth of downlink carrier in number of PRBs} if subcarrier spacing is15kHz; Minimum of {48, bandwidth of downlink carrier in number of PRBs } if subcarrier spacing is 30kHz; allowedValues: 1,2..96 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nrofGlobalRIMRSFrequencyCandidates | It is the number of candidate frequency resources in the whole network () (see 38.211 [32], clause 7.4.1.6). allowedValues: 1,2,4 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rimRSStartingFrequencyOffsetIdList | It is a list of configured frequency offsets in units of resource blocks, where each element is the frequency offset relative to a configured reference point for RIM-RS. The size of the list is nrofGlobalRIMRSFrequencyCandidates and the resulting frequency resource blocks of RIM-RS corresponding to different configured frequency offset have no overlapping bandwidth. (see 38.211 [32], clause 7.4.1.6). . allowedValues: 0..maxNrofPhysicalResourceBlocks-1 where maxNrofPhysicalResourceBlocks = 550 | type: Integer multiplicity: 1, 2, 4 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nrofRIMRSSequenceCandidatesofRS1 | It is the number of candidate sequences assigned for RIM RS-1 () (see 38.211 [32], clause 7.4.1.6). It should be even when enableEnoughNotEnoughIndication for RS-1 is ON allowedValues: 1,2..8 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rimRSScrambleIdListofRS1 | It is list of configured scrambling identities for RIM RS-1 (see 38.211 [32], clause 7.4.1.6). The size of the list is nrofRIMRSSequenceCandidatesofRS1. allowedValues: 0..2^10-1 | type: Integer multiplicity: 1, 2..8 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nrofRIMRSSequenceCandidatesofRS2 | It is the number of candidate sequences assigned for RIM RS-2 (see 38.211 [32], clause 7.4.1.6). allowedValues: 1,2..8 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rimRSScrambleIdListofRS2 | It is list of configured scrambling identities for RIM RS-2 (see 38.211 [32], clause 7.4.1.6).. The size of the list is nrofRIMRSSequenceCandidatesofRS2. allowedValues: 0..2^10-1 | type: Integer multiplicity: 1, 2..8 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| enableEnoughNotEnoughIndication | It is indication of whether "Enough" / "Not enough" indication functionality is enabled for RIM RS-1 (see 38.211 [32], clause 7.4.1.6). If the indication is "enable", the first half of nrofRIMRSSequenceCandidatesofRS1 sequences indicates "Not enough mitigation", and the second half indicates "Enough mitigation", where, "Enough mitigation" indicates that IoT going back to certain level at victim side and/or no further interference mitigation actions are needed at aggressor side "Not enough mitigation" indicates that IoT exceeding certain level at victim side and/or further interference mitigation actions are needed at aggressor side allowedValues: "ENABLE", "DISABLE" see NOTE 8 | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: DISABLE isNullable: False |
| RIMRSScrambleTimerMultiplier | It is parameter multiplier factor Z for initialization seed (see 38.211 [32], clause 7.4.1.6). allowedValues: 0,1,….2^31-1 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| RIMRSScrambleTimerOffset | It is parameter offset for initialization seed (see 38.211 [32], clause 7.4.1.6). allowedValues: 0,1,….2^31-1 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dlULSwitchingPeriod1 | This attribute is used to configure the first uplink-downlink switching period (P1) for RIM RS transmission in the network, where one RIM RS is configured in one uplink-downlink switching period. (see 38.211 [32], clause 7.4.1.6). When only one TDD-UL-DL-Pattern is configured, only dl-UL-SwitchingPeriod1 is configured, where P1 equals to the transmission periodicity of the TDD-UL-DL-Pattern. When two concatenated TDD-UL-DL-Patterns are configured, and RIM-RS resources is configured only in one of the TDD patterns, only dl-UL-SwitchingPeriod1 is configured, where P1 equals to the addition of the concatenated transmission periodicity of the two TDD-UL-DL-Patterns. When two concatenated TDD-UL-DL-Patterns are configured, and RIM-RS resources are configured in both TDD patterns, both dl-UL-SwitchingPeriod1 and dl-UL-SwitchingPeriod2 are configured, where P1 equals to the transmission periodicity of the first TDD-UL-DL-Pattern. See NOTE 6 allowedValues: MS0P5, MS0P625, MS1, MS1P25, MS2, MS2P5, MS4, MS5, MS10, MS20, if a single uplink-downlink period is configured for RIM-RS purposes; MS0P5, MS0P625, MS1, MS1P25, MS2, MS2P5, MS3, MS4, MS5, MS10, MS20, if two uplink-downlink periods are configured for RIM-RS purposes. see NOTE 9 | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| symbolOffsetOfReferencePoint1 | This attribute is used to configure the reference point in the first uplink-downlink switching period, which is the symbols offset of the reference point after the starting boundary of the first uplink-downlink switching period. It's Configured together with dl-UL-SwitchingPeriod1 (see 38.211 [32], clause 7.4.1.6). When only one TDD-UL-DL-Pattern is configured, the reference point configured for the first uplink-downlink switching period is the DL transmission boundary of the TDD-UL-DL-Pattern. When two concatenated TDD-UL-DL-Patterns are configured, and RIM-RS resources is configured only in one of the TDD patterns, the reference point configured for the first uplink-downlink switching period is the DL transmission boundary of the TDD-UL-DL-Pattern where the RIM-RS resource is configured. When two concatenated TDD-UL-DL-Patterns are configured, and RIM-RS resources are configured in both TDD patterns, the reference points configured for first uplink-downlink switching period is the DL transmission boundary of the first TDD-UL-DL-Pattern. allowedValues: 2, 3..20*2*maxNrofSymbols-1, where maxNrofSymbols=14 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dlULSwitchingPeriod2 | This attribute is used to configure the second uplink-downlink switching period (P2) for RIM RS transmission in the network, where one RIM RS is configured in one uplink-downlink switching period (see 38.211 [32], clause 7.4.1.6). When two concatenated TDD-UL-DL-Patterns are configured, and RIM-RS resources are configured in both TDD patterns, both dl-UL-SwitchingPeriod1 and dl-UL-SwitchingPeriod2 are configured, where P2 equals to the transmission periodicity of the second TDD-UL-DL-Pattern, and where (P1 + P2) divides 20 ms. allowedValues: MS0P5, MS0P625, MS1, MS1P25, MS2, MS2P5, MS3, MS4, MS5, MS10, MS20 See NOTE 9 | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| symbolOffsetOfReferencePoint2 | This attribute is used to configure the reference point in the second uplink-downlink switching period, which is the symbol offset of the reference point after starting boundary of the second uplink-downlink switching period. Configured together with dl-UL-SwitchingPeriod2 (see 38.211 [32], clause 7.4.1.6). When two concatenated TDD-UL-DL-Patterns are configured, and RIM-RS resources are configured in both TDD patterns, the reference points configured for second uplink-downlink switching period is the DL transmission boundary of the second TDD-UL-DL-Pattern. allowedValues: 2, 3..20*2*maxNrofSymbols-1, where maxNrofSymbols=14 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| totalnrofSetIdofRS1 | It is the total number of set IDs for RIM RS-1 () (see 38.211 [32], clause 7.4.1.6). allowedValues: 0,1...2^22-1 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| totalnrofSetIdofRS2 | It is the total number of set IDs for RIM RS-2 (see 38.211 [32], clause 7.4.1.6). allowedValues: 0,1...2^22-1 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nrofConsecutiveRIMRS1 | It is number of consecutive uplink-downlink switching periods for RS-1 (R1) for repetition/near-far indication:. (see 38.211 [32], clause 7.4.1.6). allowedValues: 1,2,4,8 see NOTE 7 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nrofConsecutiveRIMRS2 | It is number of consecutive uplink-downlink switching periods for RS-2 (R2) for repetition/near-far indication. (see 38.211 [32], clause 7.4.1.6). allowedValues: 1,2,4,8 see NOTE 7 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| consecutiveRIMRS1List | It is used to configure the OFDM symbol position(s) of RIM RS-1 within the uplink-downlink switching period. It is a list of symbol offset of RIM RS-1 before the reference point. The size of the list is nrofConsecutiveRIMRS1 (see 38.211 [32], clause 7.4.1.6). The resulting RIM RS-1 symbols and its reference point shall belong to the same 10ms frame. . allowedValues: 2,3..20*2*maxNrofSymbols-1, where maxNrofSymbols=14 | type: Integer multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| consecutiveRIMRS2List | It is used to configure the OFDM symbol position(s) of RIM RS-2 within the uplink-downlink switching period. It is a list of symbol offset of RIM RS-2 before the reference point. The size of the list is nrofConsecutiveRIMRS2 (see 38.211 [32], clause 7.4.1.6). The resulting RIM RS-2 symbols and its reference point shall belong to the same 10ms frame. . allowedValues: 2,3..20*2*maxNrofSymbols-1, where maxNrofSymbols=14 | type: Integer multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| enablenearfarIndicationRS1 | It is indication of whether near-far functionality is enabled for RIM RS1. If the indication is "enable", the first half of nrofConsecutiveRIMRS1 (R1) consecutive uplink-downlink switching period is for "Near" indication with R1/2 repetitions, the second half of R1 consecutive uplink-downlink switching period is for "Far" indication with R1/2 repetitions. allowedValues: "ENABLE", "DISABLE" | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: DISABLE isNullable: False |
| enablenearfarIndicationRS2 | It is indication of whether near-far functionality is enabled for RIM RS2. If the indication is "enable", the first half of nrofConsecutiveRIMRS2 (R2) consecutive uplink-downlink switching period is for "Near" indication with R2/2 repetitions, the second half of R2 consecutive uplink-downlink switching period is for "Far" indication with R2/2 repetitions. allowedValues: "ENABLE", "DISABLE" | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: DISABLE isNullable: False |
| rimRSReportConf | It is used to configure gNBs to report the all necessary information derived from the detected RIM-RS to OAM. allowedValues: Not applicable | type: RimRSReportConf multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: N/A isNullable: False |
| reportIndicator | It is used to enable or disable the RS report on a gNB. If the indication is "enable", the gNB starts to periodically report necessary information derived from the detected RIM-RS to OAM. If the indication is "disable", the gNB stops reporting. allowedValues: ENABLE, DISABLE | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: DISABLE isNullable: False |
| reportInterval | It is used to define reporting interval of a gNB in ms. allowedValues: Not applicable | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nrofRIMRSReportInfo | It is used to define the maximum number of RIMRSReportInfo in a single report. allowedValues: Not applicable | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| maxPropagationDelay | It is used to define the maximum reported OFDM symbol number for the propagation delay of the detected RIM-RS in each RIMRSReportInfo. allowedValues: 0, 1..20*2*maxNrofSymbols-1, where maxNrofSymbols=14. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rimRSReportInfoList | It represents a list (the length of the list is nrofRIMRSReportInfo) of necessary information derived from the detected RIM-RS. allowedValues: Not applicable | type: RimRSReportInfo multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: N/A isNullable: False |
| detectedSetID | This attributer indicates the Set ID of the detected RIM-RS. allowedValues: 0,1...max{totalnrofSetIdofRS1, totalnrofSetIdofRS2}. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| propagationDelay | This attributer indicates the propagation delay of the detected RIM-RS, in number of OFDM symbol. allowedValues: 0, 1.. maxPropagationDelay. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| functionalityOfRIMRS | This attributer indicates the functionality of the detected RIM-RS. If the indication of enableEnoughNotEnoughIndication is "enable", valid values are {RS2, RS1forEnoughMitigation, RS1forNotEnoughMitigation}; If the indication of enableEnoughNotEnoughIndication is "disable", valid values are {RS1, RS2}. allowedValues: RS1, RS2, RS1forEnoughMitigation, RS1forNotEnoughMitigation | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rimRSMonitoringWindowDuration | This attributer configures a duration of the monitoring window in which gNB monitors the RIM-RS, in unit of , where is the RIM-RS transmission periodicity in units of uplink-downlink switching period (see 38.211 [32], clause 7.4.1.6). This field is configured together with rimRSMonitoringInterval, rimRSMonitoringWindowStartingOffset, rimRSMonitoringOccasionInterval and rimRSMonitoringOccasionStartingOffset. The duration of the monitoring window is expected to be larger than or equal to , where is the interval between adjacent monitoring occasions within the monitoring window (configured by rimRSMonitoringInterval). The absolute duration of the monitoring window is not expected to be larger than the periodicity of the monitoring window (configured by rimRSMonitoringWindowPeriodicity). Only the earliest consecutive detection durations in each RIM-RS transmission periodicity () in the monitoring window are taken as valid time for monitoring potential interference, and they are consecutively monitored in the monitoring window, while the residual part of each RIM-RS transmission periodicity is not used for discovering potential interference, where, a consecutive detection duration spans (if only is configured) or (if both and are configured), where, is the number of consecutive uplink-downlinkswitching periods for RS-1 (configured by nrofConsecutiveRIMRS1), is the first uplink-downlinkswitching period (configured by dlULSwitchingPeriod1), is the second uplink-downlink switching period (configured by dlULSwitchingPeriod2), and is the total number of set IDs for RIM RS-1 (configured by totalnrofSetIdofRS1), is the number of candidate frequency resources in the whole network (configured by nrofGlobalRIMRSFrequencyCandidates), and is the number of candidate sequences assigned for RIM RS-1 (configured by nrofRIMRSSequenceCandidatesofRS1). allowedValues: 1,2,..2^14 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rimRSMonitoringWindowPeriodicity | This attributer configures the periodicity of the monitoring window, in unit of hours. allowedValues: 1, 2, 3, 4, 6, 8, 12, 24 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rimRSMonitoringWindowStartingOffset | This attributer configures the start offset of the first monitoring window within one day, in unit of hours. allowedValues: 0,1,2..23 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rimRSMonitoringOccasionInterval | This attributer configures the interval between adjacent monitoring occasions (M) within the monitoring window, in unit of consecutive detection duration. M is expected to be prime to , where is given in above attribute rimRSMonitoringWindowDuration. allowedValues: 1,2..-1. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rimRSMonitoringOccasionStartingOffset | This attributer configures the start offset of the first monitoring occasions within the monitoring window (), in unit of consecutive detection duration. gNB starts monitoring potential interference from the -th consecutive detection duration in the first complete RIM-RS transmission periodicity () within the monitoring window. allowedValues: 0,1,2..M-1 where M is the the interval between adjacent monitoring occasions within the monitoring window (configured by rimRSMonitoringOccasionInterval) | Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| victimSetRef | This attribute contains the DN of a victim Set (RimRSSet) allowedValues: Not applicable. | type: DN multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| aggressorSetRef | This attribute contains the DN of an aggressor Set (RimRSSet) allowedValues: Not applicable. | type: DN multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| setType | The attribute specifies type of a RIM-RS Set . RIM RS1 is transmitted by victim to indicate its suffering remote interference, and RIM RS2 is transmitted by aggressor to measure if Remote Interference still exist allowedValues: RS1, RS2. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nRCellDURef | This attribute contains the DN of a NR Cell (NRCellDU) allowedValues: Not applicable. | type: DN multiplicity: * isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| isENDCAllowed | This indicates if EN-DC is allowed or prohibited. If TRUE, the target cell is allowed to be used for EN-DC. The target cell is referenced by the NRCellRelation that contains this isENDCAllowed. If FALSE, EN-DC shall not be allowed. allowedValues: TRUE,FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| x2BlackList | This is a list of GeNBIds. If the target node GeNBId is a member of the source node's NRCellCU.x2BlackList, the source node is: 1) prohibited from sending X2 connection requests to the target node; 2) forced to tear down an established X2 connection to the target node; 3) not allowed to accept incoming X2 connection requests from the target node. The same GeNBId may appear here and in NRCellCU.x2WhiteList. In such case, the GeNBId in x2WhiteList shall be treated as if it is absent. allowedValues: See NOTE 5. | type: String multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| xnBlackList | This is a list of GgNBIds. If the target node GgNBId is a member of the source node's NRCellCU.xnBlackList, the source node is: 1) prohibited from sending Xn connection requests to the target node; 2) forced to tear down an established Xn connection to the target node; 3) not allowed to accept incoming Xn connection requests from the target node. The same GgNBId may appear here and in NRCellCU.xnWhiteList. In such case, the GgNBId in xnWhiteList shall be treated as if it is absent. allowedValues: See NOTE 5. | type: String multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| x2WhiteList | This is a list of GeNBIds. If the target node GeNBId is a member of the source node's NRCellCU.x2WhiteList, the source node is: 1) allowed to request the establishment of an X2 connection to the target node; 2) not allowed to initiate the tear down of an established X2 connection to the target node The same GeNBId may appear here and in NRCellCU.x2BlackList. In such case, the GeNBId here shall be treated as if it is absent. allowedValues: See NOTE 5. | type: String multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| xnWhiteList | This is a list of GgNBIds. If the target node GgNBId is a member of the source node's NRCellCU.xnWhiteList, the source node is: 1) allowed to request the establishment of Xn connection with the target node; 2) not allowed to initiate the tear down of an established Xn connection to the target node The same GgNBId may appear here and in NRCellCU.xnBlackList. In such case, the GgNBId here shall be treated as if it is absent. allowedValues: See NOTE 5. | type: String multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| xnHOBlackList | This is a list of GgNBIds. For all the entries in NRCellCU.xnHOBlackList, the subject NRCellCU is prohibited to use the Xn interface for HOs even if an Xn interface exists to the target cell. allowedValues: See NOTE 5. | type: String multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| x2HOBlackList | This is a list of GeNBIds. For all the entries in NRCellCU.x2HOBlackList, the subject NRCellCU is prohibited to use the X2 interface for HOs even if an X2 interface exists to the target cell. allowedValues: See NOTE 5. | type: String multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| tceIDMappingInfoList | This attribute includes a list of TCE ID, PLMN where TCE resides and the corresponding TCE IP address. It is used in Logged MDT case to provide the information to the gNodeB or GNBCUCPFunction to get the corresponding TCE IP address when there is an MDT log received from the UE. allowedValues: Not applicable | type: tceIDMappingInfo multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| tceIPAddress | This attribute indicates IP address of TCE. (See clause 4.1.1.9.2 in TS 32.422[68]) | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| tceID | This attribute indicates TCE Id. (See clause 4.1.1.9.2 in TS 32.422[68]) | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| pLMNTarget | This attribute indicates PLMN where TCE resides. (See clauses 4.1.1.9.2 and 4.9.2 in TS 32.422 [68]) | Type: PLMNId multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NOTE 1: Void NOTE 2: The radio resource can be signaling resources (e.g. RRC connected users) or user plane resources (e.g. PRB, DRB). Different RRM Policy maybe applied for different types of radio resource. E.g. RRMPolicyRatio is used for PRB resource. NOTE 3: Void NOTE 4: A RRM Policy can make use of the defined policy (e.g. RRMPolicyRatio) or a vendor specific RRM Policy. NOTE 5: For Global gNB Identifiers, the entries are formatted according to the pattern <mcc><mnc>-<gNBIdLength>-<gNBId>, where <mcc> is three digits, <mnc> two or three digits, <gNBIdLength> is a string containing a number n as digits, in the range 22 to 32, and <gNBId> is a string containing digits for the number 0 to 2n-1. For Global eNB Identifiers, the entries are formatted according to the pattern <mcc><mnc>-<eNBIdLength>-<eNBId>, where <mcc> is three digits, <mnc> two or three digits, <gNBIdLength> is a string containing a number m as digits, m being one of 18, 20, 21 or 22, and <eNBId> is a string containing digits for the number 0 to 2m-1. NOTE 6: The maximum number of total RIM RS sequence within 10ms is 32 regardless single or two uplink-downlink period are configured in the 10ms.. NOTE 7: 1. The maximum number of consecutive uplink-downlink switching periods for repetition/near-far-functionality is 8 (the number can be either 2, 4, or 8) with near-far functionality and with repetition. 2. The maximum number of consecutive uplink-downlink switching periods for repetition is 4 (the number can be either 1, 2, or 4) without near-far functionality and with repetition only. 3. The maximum number of consecutive uplink-downlink switching periods is 2 with near-far functionality only and without repetition. NOTE 8 (for information): "Not enough mitigation" means aggressor gNB needs to increase the interference mitigation level (i.e., further interference mitigation actions) (e.g., further reducing the DL transmission power on DL symbols at aggressor side), while "Enough mitigation" means aggressor gNB keeping the current interference mitigation level unchanged (i.e., no further interference mitigation actions) (e.g., remaining the DL transmission power on DL symbols unchanged at aggressor side). NOTE 9: Value MS0P5 corresponds to 0.5 ms, MS0P625 corresponds to 0.625 ms, MS1 corresponds to 1 ms, MS1P25 corresponds to 1.25 ms, and so on. |  |  |

## 4.5	Common notifications

### 4.5.1	Alarm notifications

This clause presents a list of notifications, defined in TS 28.532 [35], that an MnS consumer may receive. The notification header attribute objectClass/objectInstance shall capture the DN of an instance of a class defined in the present document.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyNewAlarm | M | -- |
| notifyClearedAlarm | M | -- |
| notifyAckStateChanged | M | -- |
| notifyAlarmListRebuilt | M | -- |
| notifyChangedAlarm | O | -- |
| notifyCorrelatedNotificationChanged | O | -- |
| notifyChangedAlarmGeneral | O | -- |
| notifyComments | O | -- |
| notifyPotentialFaultyAlarmList | O | -- |

### 4.5.2	Configuration notifications

This clause presents a list of notifications, defined in TS 28.532 [35], that an MnS consumer may receive. The notification header attribute objectClass/objectInstance shall capture the DN of an instance of a class defined in the present document.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyMOICreation | O | -- |
| notifyMOIDeletion | O | -- |
| notifyMOIAttributeValueChanges | O | -- |
| notifyEvent | O | -- |

# 5	Information Model definitions for 5GC NRM

## 5.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.622 [30], IOC, SubNetwork | SubNetwork |
| TS 28.622 [30], IOC, ManagedElement | ManagedElement |
| TS 28.622 [30], IOC, ManagedFunction | ManagedFunction |
| TS 28.622 [30], IOC, EP_RP | EP_RP |
| TS 28.708 [21], IOC, ServingGWFunction | ServingGWFunction |
| TS 28.702 [20], IOC, SmsIwmscFunction | SmsIwmscFunction |
| TS 28.702 [20], IOC, SmsGmscFunction | SmsGmscFunction |
| TS 28.702 [20], IOC, GmlcFunction | GmlcFunction |
| TS 28.658 [19], dataType, PLMNId | PLMNId |

## 5.2	Class diagram

### 5.2.1	Class diagram of 5GC NFs

#### 5.2.1.1	Relationships

This clause depicts the set of classes (e.g. IOCs) that encapsulates the information relevant for NRM of 5GC NFs definitions. This clause provides the overview of the relationships of relevant classes in UML. Subsequent clauses provide more detailed specification of various aspects of these classes.

The Figure 5.2.1.1-1 shows the 5GC NF NRM containment/naming relationship.

Figure 5.2.1.1-1: 5GC NRM containment/naming relationship

The Figure 5.2.1.1-2 shows the transport view of AMF NRM.

Figure 5.2.1.1-2: Transport view of AMF NRM

The Figure 5.2.1.1-3 shows the transport view of SMF NRM.

Figure 5.2.1.1-3: Transport view of SMF NRM

The Figure 5.2.1.1-4 shows the transport view of UPF NRM.

Figure 5.2.1.1-4: Transport view of UPF NRM

The Figure 5.2.1.1-5 shows the transport view of N3IWF NRM.

Figure 5.2.1.1-5: Transport view of N3IWF NRM

The Figure 5.2.1.1-6 shows the transport view of PCF NRM.

Figure 5.2.1.1-6: Transport view of PCF NRM

The Figure 5.2.1.1-7 shows the transport view of AUSF NRM.

Figure 5.2.1.1-7: Transport view of AUSF NRM

The Figure 5.2.1.1-8 shows the transport view of UDM NRM.

Figure 5.2.1.1-8: Transport view of UDM NRM

The Figure 5.2.1.1-9 shows the transport view of NRF NRM.

Figure 5.2.1.1-9: Transport view of NRF NRM

The Figure 5.2.1.1-10 shows the transport view of NSSF NRM.

Figure 5.2.1.1-10: Transport view of NSSF NRM

The Figure 5.2.1.1-11 shows the transport view of SMSF NRM.

Figure 5.2.1.1-11: Transport view of SMSF NRM

The Figure 5.2.1.1-12 shows the transport view of 5G location service related NRM.

Figure 5.2.1.1-12: Transport view of LMF NRM

The Figure 5.2.1.1-13 shows the transport view of 5G-EIR NRM.

Figure 5.2.1.1-13: Transport view of 5G-EIR NRM

The Figure 5.2.1.1-14 shows the transport view of SEPP NRM.

Figure 5.2.1.1-14: Transport view of SEPP NRM

The Figure 5.2.1.1-15 shows the NRM fragment for control of QoS monitoring per QoS flow per UE.

Figure 5.2.1.1-15: NRM fragment for control of QoS monitoring per QoS flow per UE

The Figure 5.2.1.1-16 shows the NRM fragment for control of GTP-U path QoS monitoring.

Figure 5.2.1.1-16: NRM fragment for control of GTP-U path QoS monitoring

The Figure 5.2.1.1-17 shows the NRM fragment for configurable 5QIs in 5GC.

Figure 5.2.1.1-17: NRM fragment for configurable 5QIs in 5GC

The Figure 5.2.1.1-18 shows the NRM fragment for 5QI and DSCP mapping.

Figure 5.2.1.1-18: NRM fragment for 5QI and DSCP mapping.

The Figure 5.2.1.1-19 shows the NRM fragment for predefined PCC rule.

Figure 5.2.1.1-19: NRM fragment for predefined PCC rule

The Figure 5.2.1.1-20 shows the NRM fragment for dynamic 5QIs in 5GC.

Figure 5.2.1.1-20: NRM fragment for dynamic 5QIs in 5GC

#### 5.2.1.2	Inheritance

This clause depicts the inheritance relationships that exist between IOCs.

Figure 5.2.1.2-1 shows the inheritance hierarchy from IOC ManagedFunction related to the 5GC NF NRM.

Figure 5.2.1.2-1: Inheritance hierarchy from IOC ManagedFunction related to the 5GC NF NRM

Figure 5.2.1.2-2 shows the inheritance hierarchy from IOC EP_RP related to 5GC NF NRM.

Figure 5.2.1.2-2: Inheritance hierarchy from IOC EP_RP related to the 5GC NF NRM

Figure 5.2.1.2-3: Inheritance hierarchy for IOC QFQoSMonitoringControl

Figure 5.2.1.2-4: Inheritance hierarchy for IOC GtpUPathQoSMonitoringControl

Figure 5.2.1.2-5: Inheritance hierarchy for IOC Configurable5QISet

Figure 5.2.1.2-6: Inheritance hierarchy for IOC FiveQiDscpMapping

Figure 5.2.1.2-7: Inheritance hierarchy for predefined PCC rule modeling

Figure 5.2.1.2-8: Inheritance hierarchy for IOC Dynamic5QISet

### 5.2.2	Class diagram of AMF Region/AMF Set

#### 5.2.2.1	Relationships

This clause depicts the set of classes (e.g. IOCs) that encapsulates the information relevant for NRM of AMF Region/AMF Set definitions. This clause provides the overview of the relationships of relevant classes in UML. Subsequent clauses provide more detailed specification of various aspects of these classes.

The Figure 5.2.2.1-1 shows the AMF Region/AMF Set NRM containment/naming relationship.

Figure 5.2.2.1-1: AMF Region/AMF Set NRM

#### 5.2.2.2	Inheritance

This clause depicts the inheritance relationships that exist between IOCs.

Figure 5.2.2.2-1 shows the inheritance hierarchy from IOC ManagedFunction related to the AMF Region/AMF Set NRM.

Figure 5.2.2.2-1: AMF Region/AMF Set Inheritance

## 5.3	Class definitions

### 5.3.1	AMFFunction

#### 5.3.1.1	Definition

This IOC represents the AMF functionality in 5GC. For more information about the AMF, see TS 23.501 [2].

#### 5.3.1.2	Attributes

The AMFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| aMFIdentifier | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| sNSSAIList | CM | T | F | F | T |
| managedNFProfile | M | T | T | F | T |
| commModelList | M | T | T | F | T |

#### 5.3.1.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: Network slicing feature is supported. |

#### 5.3.1.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.2	SMFFunction

#### 5.3.2.1	Definition

This IOC represents the SMF function in 5GC. For more information about the SMF, see TS 23.501 [2].

#### 5.3.2.2	Attributes

The SMFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| nRTAClist | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| commModelList | M | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| configurable5QISetRef | O | T | T | F | T |
| dynamic5QISetRef | O | T | F | F | T |

#### 5.3.2.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: Network slicing feature is supported. |

#### 5.3.2.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.3	UPFFunction

#### 5.3.3.1	Definition

This IOC represents the UPF function in 5GC. For more information about the UPF, see TS 23.501 [2].

#### 5.3.3.2	Attributes

The UPFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| nRTAClist | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| supportedBMOList | O | T | T | F | T |

#### 5.3.3.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList CM Support Qualifier | The condition is "network slicing feature is supported". |

#### 5.3.3.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.4	N3IWFFunction

#### 5.3.4.1	Definition

This IOC represents the N3IWF function which is used to enable non-3GPP access networks connected to the 5GC. For more information about the N3IWF, see TS 23.501 [2].

#### 5.3.4.2	Attributes

The N3IWFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| commModelList | M | T | T | F | T |

#### 5.3.4.3	Attribute constraints

None.

#### 5.3.4.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.5	PCFFunction

#### 5.3.5.1	Definition

This IOC represents the PCF function in 5GC. For more information about the PCF, see TS 23.501 [2].

#### 5.3.5.2	Attributes

The PCFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| commModelList | M | T | T | F | T |
| supportedBMOList | O | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| configurable5QISetRef | O | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| dynamic5QISetRef | O | T | F | F | T |

#### 5.3.5.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: network slicing feature is supported. |

#### 5.3.5.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.6	AUSFFunction

#### 5.3.6.1	Definition

This IOC represents the AUSF function in 5GC. For more information about the AUSF, see TS 23.501 [2].

#### 5.3.6.2	Attributes

The AUSFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| commModelList | M | T | T | F | T |

#### 5.3.6.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: Network slicing feature is supported. |

#### 5.3.6.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.7	UDMFunction

#### 5.3.7.1	Definition

This IOC represents the UDM function in 5GC. For more information about the UDM, see TS 23.501 [2].

#### 5.3.7.2	Attributes

The UDMFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| commModelList | M | T | T | F | T |

#### 5.3.5.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: network slicing feature is supported. |

#### 5.3.5.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.8	UDRFunction

#### 5.3.8.1	Definition

This IOC represents the UDR function in 5GC. For more information about the UDR, see TS 23.501 [2].

#### 5.3.8.2	Attributes

The UDRFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| managedNFProfile | M | T | T | F | T |

#### 5.3.8.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: Network slicing feature is supported. |

#### 5.3.8.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.9	UDSFFunction

#### 5.3.9.1	Definition

This IOC represents the UDSF function which can be interacted with any other 5GC NF defined in TS 23.501 [2]. For more information about the UDSF, see TS 23.501 [2].

#### 5.3.9.2	Attributes

The UDSFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| managedNFProfile | M | T | T | F | T |

#### 5.3.9.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: Network slicing feature is supported. |

#### 5.3.9.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.10	NRFFunction

#### 5.3.10.1	Definition

This IOC represents the NRF function in 5GC. For more information about the NRF, see TS 23.501 [2].

#### 5.3.10.2	Attributes

The NRFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| nFProfileList | CM | T | T | F | T |
| cNSIIdList | O | T | T | F | T |

#### 5.3.10.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: network slicing feature is supported. |
| nfProfileList Support Qualifier | Condition: NF profile is registered and deregistered by management system. |
| nSIIdList Support Qualifier | Condition: Network slicing feature is supported. |

#### 5.3.10.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.11	NSSFFunction

#### 5.3.11.1	Definition

This IOC represents the NSSF function in 5GC. For more information about the NSSF, see TS 23.501 [2].

#### 5.3.11.2	Attributes

The NSSFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| sNSSAIList | M | T | T | F | T |
| cNSIIdList | O | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| commModelList | M | T | T | F | T |

#### 5.3.11.3	Attribute constraints

None.

#### 5.3.11.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.12	AFFunction

#### 5.3.12.1	Definition

This IOC is defined for only purpose to describe the IOCs representing its interaction interface with 5GC (i.e. EP_Rx and EP_N5). It has no any attributes defined.

### 5.3.13	DNFunction

#### 5.3.13.1	Definition

This IOC is defined for only purpose to describe the IOCs representing Data Network (DN) interaction interface with 5GC (i.e. EP_N6). It has no any attributes defined.

### 5.3.14	SMSFFunction

#### 5.3.14.1	Definition

This IOC represents the SMSF function defined in TS 23.501 [2].

#### 5.3.14.2	Attributes

The SMSFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| commModelList | M | T | T | F | T |

#### 5.3.14.3	Attribute constraints

None.

#### 5.3.14.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.15	LMFFunction

#### 5.3.15.1	Definition

This IOC represents the LMF function defined in TS 23.501 [2].

#### 5.3.15.2	Attributes

The LMFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| commModelList | M | T | T | F | T |

#### 5.3.15.3	Attribute constraints

None.

#### 5.3.15.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.16	NGEIRFunction

#### 5.3.16.1	Definition

This IOC represents the 5G-EIR function in 5GC. For more information about the 5G-EIR, see TS 23.501 [2].

#### 5.3.16.2	Attributes

The NGEIRFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| commModelList | M | T | T | F | T |

#### 5.3.16.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: network slicing feature is supported. |

#### 5.3.16.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.17	SEPPFunction

#### 5.3.17.1	Definition

This IOC represents the SEPP function which support message filtering and policing on inter-PLMN control plane interface. For more information about the SEPP, see TS 23.501 [2].

#### 5.3.17.2	Attributes

The SEPPFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNId | M | T | F | T | T |
| sEPPType | M | T | F | T | T |
| sEPPId | M | T | F | T | T |
| fqdn | M | T | T | F | T |

#### 5.3.17.3	Attribute constraints

None.

#### 5.3.17.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.18	NWDAFFunction

#### 5.3.18.1	Definition

This IOC represents the NWDAF function in 5GC. For more information about the NWDAF, see TS 23.501 [2].

#### 5.3.18.2	Attributes

The NWDAFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| commModelList | M | T | T | F | T |

#### 5.3.18.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: Network slicing feature is supported. |

#### 5.3.18.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.19	EP_N2

#### 5.3.19.1	Definition

This IOC represents the N2 interface between (R)AN and AMF, which is defined in TS 23.501 [2].

#### 5.3.19.2	Attributes

The EP_N2 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.19.3	Attribute constraints

None.

#### 5.3.19.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.20	EP_N3

#### 5.3.20.1	Definition

This IOC represents the N3 interface between (R)AN and UPF, which is defined in TS 23.501 [2].

#### 5.3.20.2	Attributes

The EP_N3 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| epTransportRef | O | T | F | F | T |

#### 5.3.20.3	Attribute constraints

None.

#### 5.3.20.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.21	EP_N4

#### 5.3.21.1	Definition

This IOC represents the N4 interface between SMF and UPF, which is defined in TS 23.501 [2].

#### 5.3.21.2	Attributes

The EP_N4 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.21.3	Attribute constraints

None.

#### 5.3.21.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.22	EP_N5

#### 5.3.22.1	Definition

This IOC represents the N5 interface between PCF and AF, which is defined in TS 23.501 [2].

#### 5.3.22.2	Attributes

The EP_N5 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.22.3	Attribute constraints

None.

#### 5.3.22.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.23	EP_N6

#### 5.3.23.1	Definition

This IOC represents the N6 interface between UPF and DN, which is defined in TS 23.501 [2].

#### 5.3.23.2	Attributes

The EP_N6 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.23.3	Attribute constraints

None.

#### 5.3.23.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.24	EP_N7

#### 5.3.24.1	Definition

This IOC represents the N7 interface between SMF and PCF, which is defined in TS 23.501 [2].

#### 5.3.24.2	Attributes

The EP_N7 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.24.3	Attribute constraints

None.

#### 5.3.24.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.25	EP_N8

#### 5.3.25.1	Definition

This IOC represents the N8 interface between AMF and UDM, which is defined in TS 23.501 [2].

#### 5.3.25.2	Attributes

The EP_N8 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.25.3	Attribute constraints

None.

#### 5.3.25.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.26	EP_N9

#### 5.3.26.1	Definition

This IOC represents the N7 interface between two UPFs, which is defined in TS 23.501 [2].

#### 5.3.26.2	Attributes

The EP_N9 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.26.3	Attribute constraints

None.

#### 5.3.26.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.27	EP_N10

#### 5.3.27.1	Definition

This IOC represents the N10 interface between SMF and UDM, which is defined in TS 23.501 [2].

#### 5.3.27.2	Attributes

The EP_N10 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.27.3	Attribute constraints

None.

#### 5.3.27.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.28	EP_N11

#### 5.3.28.1	Definition

This IOC represents the N11 interface between AMF and SMF, which is defined in TS 23.501 [2].

#### 5.3.28.2	Attributes

The EP_N11 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.28.3	Attribute constraints

None.

#### 5.3.28.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.29	EP_N12

#### 5.3.29.1	Definition

This IOC represents the N12 interface between AMF and AUSF, which is defined in TS 23.501 [2].

#### 5.3.29.2	Attributes

The EP_N12 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.29.3	Attribute constraints

None.

#### 5.3.29.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.30	EP_N13

#### 5.3.30.1	Definition

This IOC represents the N13 interface between AUSF and UDM, which is defined in TS 23.501 [2].

#### 5.3.30.2	Attributes

The EP_N13 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.30.3	Attribute constraints

None.

#### 5.3.30.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.31	EP_N14

#### 5.3.31.1	Definition

This IOC represents the N14 interface between two AMFs, which is defined in TS 23.501 [2].

#### 5.3.31.2	Attributes

The EP_N14 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.31.3	Attribute constraints

None.

#### 5.3.31.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.32	EP_N15

#### 5.3.32.1	Definition

This IOC represents the N15 interface between AMF and PCF, which is defined in TS 23.501 [2].

#### 5.3.32.2	Attributes

The EP_N15 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.32.3	Attribute constraints

None.

#### 5.3.32.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.33	EP_N16

#### 5.3.33.1	Definition

This IOC represents the N16 interface between two SMFs, which is defined in TS 23.501 [2].

#### 5.3.33.2	Attributes

The EP_N16 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.33.3	Attribute constraints

None.

#### 5.3.33.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.34	EP_N17

#### 5.3.34.1	Definition

This IOC represents the N17 interface between AMF and 5G-EIR, which is defined in TS 23.501 [2].

#### 5.3.34.2	Attributes

The EP_N17 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.34.3	Attribute constraints

None.

#### 5.3.34.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.35	EP_N20

#### 5.3.35.1	Definition

This IOC represents the N20 interface between AMF and SMSF, which is defined in TS 23.501 [2].

#### 5.3.35.2	Attributes

The EP_N20 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.35.3	Attribute constraints

None.

#### 5.3.35.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.36	EP_N21

#### 5.3.36.1	Definition

This IOC represents the N21 interface between SMSF and UDM, which is defined in TS 23.501 [2].

#### 5.3.36.2	Attributes

The EP_N21 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.36.3	Attribute constraints

None.

#### 5.3.36.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.37	EP_N22

#### 5.3.37.1	Definition

This IOC represents the N22 interface between AMF and NSSF, which is defined in TS 23.501 [2].

#### 5.3.37.2	Attributes

The EP_N22 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.37.3	Attribute constraints

None.

#### 5.3.37.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.38	EP_N26

#### 5.3.38.1	Definition

This IOC represents the N26 interface between AMF and MME, which is defined in TS 23.501 [2].

#### 5.3.38.2	Attributes

The EP_N26 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.38.3	Attribute constraints

None.

#### 5.3.38.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.39	Void

### 5.3.40	Void

### 5.3.41	EP_S5C

#### 5.3.41.1	Definition

This IOC represents the S5-C interface between SGW and SMF/PGW-C, which is defined in TS 23.501 [2].

#### 5.3.41.2	Attributes

The EP_S5C IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.41.3	Attribute constraints

None.

#### 5.3.41.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.42	EP_S5U

#### 5.3.42.1	Definition

This IOC represents the S5-U interface between SGW and UPF/PGW-U, which is defined in TS 23.501 [2].

#### 5.3.42.2	Attributes

The EP_S5U IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.42.3	Attribute constraints

None.

#### 5.3.42.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.43	EP_Rx

#### 5.3.43.1	Definition

This IOC represents the Rx interface between PCF and AF, which is defined in TS 23.501 [2].

#### 5.3.43.2	Attributes

The EP_Rx IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.43.3	Attribute constraints

None.

#### 5.3.43.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.44	EP_MAP_SMSC

#### 5.3.44.1	Definition

This IOC represents the MAP interface between SMSF and MSC-IWMSC/GMSC, which is defined in TS 23.040 [22].

#### 5.3.44.2	Attributes

The EP_MAP_SMSC IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.44.3	Attribute constraints

None.

#### 5.3.44.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.45	EP_NLS

#### 5.3.45.1	Definition

This IOC represents the NLs interface between AMF and LMF, which is defined in TS 23.501 [2].

#### 5.3.45.2	Attributes

The EP_NLS IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.45.3	Attribute constraints

None.

#### 5.3.45.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.46	EP_NLG

#### 5.3.46.1	Definition

This IOC represents the NLg interface between AMF and GMLC, which is defined in TS 23.501 [2].

#### 5.3.46.2	Attributes

The EP_NLG IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.46.3	Attribute constraints

None.

#### 5.3.46.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.47	EP_N27

#### 5.3.47.1	Definition

This IOC represents an end point of N27 interface between vNRF and hNRF, which is defined in TS 29.510 [10].

#### 5.3.47.2	Attributes

The EP_N27 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.47.3	Attribute constraints

None.

#### 5.3.47.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.48	EP_N31

#### 5.3.48.1	Definition

This IOC represents an end point of N31 interface between vNSSF and hNSSF, which is defined in TS 29.531 [11].

#### 5.3.48.2	Attributes

The EP_N31 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.48.3	Attribute constraints

None.

#### 5.3.48.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.49	ExternalNRFFunction

#### 5.3.49.1	Definition

This IOC represents external NRF function controlled by another management domain. For more information about the NRF, see TS 23.501 [2].

#### 5.3.49.2	Attributes

The ExternalNRFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| id | M | T | F | F | T |
| pLMNIdList | M | T | T | F | T |

#### 5.3.49.3	Attribute constraints

None.

#### 5.3.49.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.50	ExternalNSSFFunction

#### 5.3.50.1	Definition

This IOC represents external NSSF function controlled by another management domain. For more information about the NSSF, see TS 23.501 [2].

#### 5.3.50.2	Attributes

The ExternalNSSFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| id | M | T | F | F | T |
| pLMNIdList | M | T | T | F | T |

#### 5.3.50.3	Attribute constraints

None.

#### 5.3.50.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.51	AMFSet

#### 5.3.51.1	Definition

This IOC represents the AMF Set which consists of some AMFs that serve a given area and Network Slice. For more information about the AMF Set, see TS 23.501 [2].

#### 5.3.51.2	Attributes

The AMFSet IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| nRTAClist | M | T | T | F | T |
| aMFSetId | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| aMFRegion | M | T | T | F | T |
| aMFSetMemberList | M | T | T | F | T |

#### 5.3.51.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: Network slicing feature is supported. |

#### 5.3.51.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.52	AMFRegion

#### 5.3.52.1	Definition

This IOC represents the AMF Region which consists one or multiple AMF Sets. For more information about the AMF Region, see TS 23.501 [2].

#### 5.3.52.2	Attributes

The AMFRegion IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| nRTAClist | M | T | T | F | T |
| aMFRegionId | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| aMFSet | M | T | T | F | T |

#### 5.3.52.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: Network slicing feature is supported. |

#### 5.3.52.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.53	ExternalAMFFunction

#### 5.3.53.1	Definition

This IOC represents an external AMF functionality used in EN-DC. For more information about the AMF, see TS 23.501 [2].

#### 5.3.53.2	Attributes

The ExternalAMFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| Id | M | T | F | F | T |
| pLMNIdList | M | T | T | F | T |
| aMFIdentifier | M | T | T | F | T |

#### 5.3.53.3	Attribute constraints

None.

#### 5.3.53.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.54	ManagedNFProfile <<dataType>>

#### 5.3.54.1	Definition

This data type represents a Profile definition of a Managed NF (See TS 23.501 [22]).

#### 5.3.54.2	Attributes

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nfInstanceID | M | T | F | T | F |
| nfType | M | T | F | F | F |
| hostAddr | M | T | T | F | T |
| authzInfo | O | T | T | F | T |
| location | O | T | T | F | T |
| capacity | O | T | T | F | T |
| nFInfo | M | T | T | F | T |

#### 5.3.54.3	Attribute constraints

None.

#### 5.3.54.4	Notifications

The clause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.55	HostAddr <<choice>>

#### 5.3.55.1	Definition

This <<choice>> stereotype represents one of a set of data types as shown in Figure 5.3.55.1-1: HostAddr <<choice>> for data types.

Figure 5.3.55.1-1: HostAddr <<choice>> for data types

NOTE: The IpAddress can be IPv4 address (See RFC 791 [24]) or IPv6 address (See RFC 2373 [25]). Refer TS 23.003 [5] for Fqdn.

### 5.3.56	NFInfo <<choice>>

#### 5.3.56.1	Definition

This <<choice>> stereotype represents one of a set of data types as shown in Figure 5.3.56.1-1: NFInfo <<choice>> for data types.

Figure 5.3.56.1-1: NFInfo choice for data types

NOTE: The AmfInfo <<dataType>> is chosed for AFMFunction, the UdrInfo <<dataType>> is chosed for UDRFunction, the UdmInfo <<dataType>> is chosed for UDMFunction, the AusfInfo<<dataType>> is chosed for AUSFFunction, the UpfInfo <<dataType>> is chosed for UPFFunction

### 5.3.57	UdmInfo <<dataType>>

#### 5.3.57.1	Definition

This data type represents a generic NFProfile definition (See TS 23.501 [22]).

#### 5.3.57.2	Attributes

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nFSrvGroupId | M | T | F | F | F |

#### 5.3.57.3	Attribute constraints

None

#### 5.3.57.4	Notifications

The clause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.58	AusfInfo <<dataType>>

#### 5.3.58.1	Definition

This data type represents a generic NFProfile definition (See TS 23.501 [22]).

#### 5.3.58.2	Attributes

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nFSrvGroupId | M | T | F | T | F |

#### 5.3.58.3	Attribute constraints

None.

#### 5.3.58.4	Notifications

The clause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.59	UpfInfo <<dataType>>

#### 5.3.59.1	Definition

This data type represents a generic NFProfile definition (See TS 23.501 [22]).

#### 5.3.59.2	Attributes

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| smfServingAreas | O | T | T | F | T |

#### 5.3.59.3	Attribute constraints

None.

#### 5.3.59.4	Notifications

The clause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.60	AmfInfo <<dataType>>

#### 5.3.60.1	Definition

This data type represents AMF specific data in NFProfile definition (See TS 23.501 [22]).

#### 5.3.60.2	Attributes

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| priority | O | T | T | F | T |

#### 5.3.60.3	Attribute constraints

None.

#### 5.3.60.4	Notifications

The clause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.61	Udrinfo <<dataType>>

#### 5.3.61.1	Definition

This data type represents UDR specific data in NFProfile definition (See TS 23.501 [22]).

#### 5.3.61.2	Attributes

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| supportedDataSetIds | O | T | T | F | T |
| nFSrvGroupId | O | T | T | F | T |

#### 5.3.61.3	Attribute constraints

None.

#### 5.3.61.4	Notifications

The clause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.62	EP_N32

#### 5.3.62.1	Definition

This IOC represents an end point of N32 interface between cSEPP and pSEPP, which is defined in TS 23.501 [2] and 33.501 [52].

#### 5.3.62.2	Attributes

The EP_N32 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| remotePlmnId | M | T | T | F | T |
| remoteSeppAddress | M | T | T | F | T |
| remoteSeppId | O | T | T | F | T |
| n32cParas | O | T | T | F | T |
| n32fPolicy | O | T | T | F | T |
| withIPX | M | T | T | F | T |

#### 5.3.62.3	Attribute constraints

None.

#### 5.3.62.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.63	ExternalSEPPFunction

#### 5.3.63.1	Definition

This IOC represents the properties, known by the management function, of a SEPP managed by another management function. For more information about SEPPFunction, see clause 5.3.17.

#### 5.3.63.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNId | M | T | F | F | T |
| sEPPId | M | T | F | T | T |
| fqdn | M | T | F | F | T |

#### 5.3.63.3	Attribute constraints

None.

#### 5.3.63.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.64	SEPPFunction <<ProxyClass>>

#### 5.3.64.1	Definition

This IOC represents an <<IOC>>SEPPFunction and <<IOC>>ExternalSEPPFunction.

#### 5.3.64.2	Attributes

See that defined in <<IOC>>SEPPFunction and <<IOC>>ExternalSEPPFunction.

#### 5.3.64.3	Attribute constraints

See respective IOCs.

#### 5.3.64.4	Notifications

See respective IOCs.

### 5.3.65	NEFFunction

#### 5.3.65.1	Definition

This IOC represents the NEF function in 5GC. For more information about the NEF, see TS 23.501 [2].

#### 5.3.65.2	Attributes

The NEFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| sBIFQDN | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| capabilityList | M | T | T | F | T |
| isINEF | O | T | F | T | F |
| isCAPIFSup | M | T | F | T | F |

#### 5.3.65.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: Network slicing feature is supported. |

#### 5.3.65.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.66	SCPFunction

#### 5.3.67.1	Definition

This IOC represents a Service Communication Proxy, which is defined in TS 23.501 [2].

#### 5.3.67.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| supportedFuncList | M | T | T | F | T |
| address | M | T | T | F | T |

#### 5.3.67.3	Attribute constraints

None.

#### 5.3.67.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.68	SupportedFunction <<dataType>>

#### 5.3.68.1	Definition

This dataType represents a functionality supported by a SCP, which is defined in TS 23.501 [2].

#### 5.3.68.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| function | M | T | T | F | T |
| policy | O | T | T | F | T |

#### 5.3.68.3	Attribute constraints

None.

#### 5.3.68.4	Notifications

The clause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.69	CommModel <<dataType>>

#### 5.3.69.1	Definition

This data type represents a communication model definition (See TS 23.501 [22]).

#### 5.3.69.2	Attributes

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| groupId | M | T | T | F | T |
| commModelType | M | T | T | F | T |
| targetNFServiceList | M | T | T | F | T |
| commModelConfiguration | M | T | T | F | T |

#### 5.3.69.3	Attribute constraints

None

#### 5.3.69.4	Notifications

The clause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.70	QFQoSMonitoringControl

#### 5.3.70.1	Definition

This IOC specifies the capabilities and properties for control of QoS monitoring per QoS flow per UE for URLLC service. For more information about QoS monitoring per QoS flow per UE, see TS 23.501 [2].

If the QoS monitoring per QoS flow per UE is enabled, the SMF requests the PSA UPF to perform the QoS monitoring per QoS flow per UE based on the attributes of the instance of this IOC.

#### 5.3.70.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| qFQoSMonitoringState | M | T | T | F | T |
| qFMonitoredSNSSAIs | M | T | T | F | T |
| qFMonitored5QIs | M | T | T | F | T |
| isEventTriggeredQFMonitoringSupported | M | T | F | F | T |
| isPeriodicQFMonitoringSupported | M | T | F | F | T |
| isSessionReleasedQFMonitoringSupported | M | T | F | F | T |
| qFPacketDelayThresholds | CM | T | T | F | T |
| qFMinimumWaitTime | CM | T | T | F | T |
| qFMeasurementPeriod | CM | T | T | F | T |

#### 5.3.70.3	Attribute constraints

| Name | Definition |
| --- | --- |
| qFPacketDelayThresholds Support Qualifier | Condition: isEventTriggeredQFMonitoringSupported attribute of the same MOI is set to "Yes". |
| qFMinimumWaitTime Support Qualifier | Condition: isEventTriggeredQFMonitoringSupported attribute of the same MOI is set to "Yes". |
| qFMeasurementPeriod Support Qualifier | Condition: isPeriodicQFMonitoringSupported attribute of the same MOI is set to "Yes". |

#### 5.3.70.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.71	QFDelayThresholdsType <<dataType>>

#### 5.3.71.1	Definition

This data type specifies the thresholds for reporting the packet delay for QoS monitoring per QoS flow per UE, see TS 29.244 [56].

#### 5.3.71.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| thresholdDl | M | T | T | F | T |
| thresholdUl | M | T | T | F | T |
| thresholdRtt | M | T | T | F | T |

#### 5.3.71.3	Attribute constraints

None

#### 5.3.71.4	Notifications

The clause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.72	GtpUPathQoSMonitoringControl

#### 5.3.72.1	Definition

This IOC specifies the capabilities and properties for control of GTP-U path QoS monitoring. For more information about the GTP-U path QoS monitoring, see TS 23.501 [2].

If the GTP-U path QoS monitoring is enabled, the SMF requests the UPF(s) and NG-RAN to perform the GTP-U path QoS monitoring based on the attributes of the instance of this IOC.

#### 5.3.72.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gtpUPathQoSMonitoringState | M | T | T | F | T |
| gtpUPathMonitoredSNSSAIs | M | T | T | F | T |
| monitoredDSCPs | M | T | T | F | T |
| isEventTriggeredGtpUPathMonitoringSupported | M | T | F | F | T |
| isPeriodicGtpUMonitoringSupported | M | T | F | F | T |
| isImmediateGtpUMonitoringSupported | M | T | F | F | T |
| gtpUPathDelayThresholds | CM | T | T | F | T |
| gtpUPathMinimumWaitTime | CM | T | T | F | T |
| gtpUPathMeasurementPeriod | CM | T | T | F | T |

#### 5.3.72.3	Attribute constraints

| Name | Definition |
| --- | --- |
| gtpUPathDelayThresholds Support Qualifier | Condition: isEventTriggeredGtpUPathMonitoringSupported attribute of the same MOI is set to "Yes". |
| gtpUPathMinimumWaitTime Support Qualifier | Condition: isEventTriggeredGtpUPathMonitoringSupported attribute of the same MOI is set to "Yes". |
| gtpUPathMeasurementPeriod Support Qualifier | Condition: isPeriodicGtpUMonitoringSupported attribute of the same MOI is set to "Yes". |

#### 5.3.72.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.73	GtpUPathDelayThresholdsType <<dataType>>

#### 5.3.73.1	Definition

This data type specifies the thresholds for reporting the packet delay for GTP-U path QoS monitoring, see TS 29.244 [56].

#### 5.3.73.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| n3AveragePacketDelayThreshold | M | T | T | F | T |
| n3MinPacketDelayThreshold | M | T | T | F | T |
| n3MaxPacketDelayThreshold | M | T | T | F | T |
| n9AveragePacketDelayThreshold | M | T | T | F | T |
| n9MinPacketDelayThreshold | M | T | T | F | T |
| n9MaxPacketDelayThreshold | M | T | T | F | T |

#### 5.3.73.3	Attribute constraints

None

#### 5.3.73.4	Notifications

The clause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.75	Configurable5QISet

#### 5.3.75.1	Definition

This IOC specifies the non-standardized 5QIs, including their QoS characteristics, that need to be pre-configured (and configurable) to the 5G NFs, see TS 23.501 [2].

#### 5.3.75.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| configurable5QIs | M | T | T | F | T |

#### 5.3.75.3	Attribute constraints

None.

#### 5.3.75.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.76	FiveQICharacteristics <<dataType>>

#### 5.3.76.1	Definition

This data type specifies the 5QI value and the cooresponding QoS characteristics for a 5QI.

#### 5.3.76.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| fiveQIValue | M | T | T/F (NOTE ) | F | T |
| resourceType | M | T | T/F (NOTE ) | F | T |
| priorityLevel | O | T | T/F (NOTE ) | F | T |
| packetDelayBudget | O | T | T/F (NOTE ) | F | T |
| packetErrorRate | O | T | T/F (NOTE ) | F | T |
| averagingWindow | O | T | T/F (NOTE ) | F | T |
| maximumDataBurstVolume | O | T | T/F (NOTE ) | F | T |
| NOTE: The isWritable qualifier is "T" if the attribute describes a configurable 5QI, and is "F" if the attribute describes a dynamic 5QI. |  |  |  |  |  |

#### 5.3.76.3	Attribute constraints

None

#### 5.3.76.4	Notifications

The clause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.77	PacketErrorRate <<dataType>>

#### 5.3.77.1	Definition

This data type specifies the Packet Error Rate of a configurable 5QI.

#### 5.3.77.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| scalar | M | T | T/F (NOTE) | F | T |
| exponent | M | T | T/F (NOTE) | F | T |
| NOTE: The isWritable qualifier is "T" if the attribute describes a configurable 5QI, and is "F" if the attribute describes a dynamic 5QI. |  |  |  |  |  |

#### 5.3.77.3	Attribute constraints

None

#### 5.3.77.4	Notifications

The clause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.78	FiveQiDscpMappingSet

#### 5.3.78.1	Definition

This IOC specifies the set of mapping between 5QIs and DSCP.

#### 5.3.78.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| fiveQiDscpMappingList | M | T | T | F | T |

#### 5.3.78.3	Attribute constraints

None.

#### 5.3.78.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.79	FiveQiDscpMapping <<dataType>>

#### 5.3.79.1	Definition

This data type specifies the mapping between 5QIs to DSCP.

#### 5.3.79.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| fiveQIValues | M | T | T | F | T |
| dscp | M | T | T | F | T |

#### 5.3.79.3	Attribute constraints

None

#### 5.3.79.4	Notifications

The clause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.80	PredefinedPccRuleSet

#### 5.3.80.1	Definition

This IOC specifies the predefined PCC rules, which are configured to SMF and referenced by PCF, see TS 23.503 [59].

#### 5.3.80.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| predefinedPccRules | M | T | T | F | T |

#### 5.3.80.3	Attribute constraints

None.

#### 5.3.80.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.81	PccRule <<dataType>>

#### 5.3.81.1	Definition

This data type specifies the PCC rule, see TS 29.512 [60].

#### 5.3.81.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pccRuleId | M | T | T | F | T |
| flowInfoList | CM | T | T | F | T |
| applicationId | CM | T | T | F | T |
| appDescriptor | O | T | T | F | T |
| contentVersion | O | T | T | F | T |
| precedence | CM | T | T | F | T |
| afSigProtocol | O | T | T | F | T |
| isAppRelocatable | O | T | T | F | T |
| isUeAddrPreserved | O | T | T | F | T |
| qosData | M | T | T | F | T |
| altQosParams | O | T | T | F | T |
| trafficControlData | M | T | T | F | T |
| conditionData | O | T | T | F | T |
| tscaiInputUl | O | T | T | F | T |
| tscaiInputDl | O | T | T | F | T |

#### 5.3.81.3	Attribute constraints

| Name | Definition |
| --- | --- |
| flowInfoList Support Qualifier | Condition: The applicationId is not supported. |
| applicationId Support Qualifier | Condition: The flowInfoList is not supported. |
| precedence Support Qualifier | Condition: The flowInfoList is provided. |

#### 5.3.81.4	Notifications

The clause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.82	FlowInformation <<dataType>>

#### 5.3.82.1	Definition

This data type specifies the flow information of a PCC rule.

#### 5.3.82.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| flowDescription | M | T | T | F | T |
| ethFlowDescription | M | T | T | F | T |
| packFiltId | M | T | T | F | T |
| packetFilterUsage | M | T | T | F | T |
| tosTrafficClass | M | T | T | F | T |
| spi | M | T | T | F | T |
| flowLabel | O | T | T | F | T |
| flowDirection | M | T | T | F | T |

#### 5.3.82.3	Attribute constraints

None

#### 5.3.82.4	Notifications

The clause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.83	EthFlowDescription <<dataType>>

#### 5.3.83.1	Definition

This data type describes an Ethernet flow.

#### 5.3.83.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| destMacAddr | M | T | T | F | T |
| ethType | M | T | T | F | T |
| fDesc | CM | T | T | F | T |
| fDir | M | T | T | F | T |
| sourceMacAddr | M | T | T | F | T |
| vlanTags | M | T | T | F | T |
| srcMacAddrEnd | O | T | T | F | T |
| destMacAddrEnd | O | T | T | F | T |

#### 5.3.83.3	Attribute constraints

| Name | Definition |
| --- | --- |
| fDesc Support Qualifier | Condition: The ethType is IP. |

#### 5.3.83.4	Notifications

The clause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.84	QoSData <<dataType>>

#### 5.3.84.1	Definition

This data type specifies the QoS control policy data for a service flow of a PCC rule.

#### 5.3.84.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| qosId | M | T | T | F | T |
| fiveQIValue | M | T | T | F | T |
| maxbrUl | O | T | T | F | T |
| maxbrDl | O | T | T | F | T |
| gbrUl | O | T | T | F | T |
| gbrDl | O | T | T | F | T |
| arp | M | T | T | F | T |
| qosNotificationControl | O | T | T | F | T |
| reflectiveQos | O | T | T | F | T |
| sharingKeyDl | O | T | T | F | T |
| sharingKeyUl | O | T | T | F | T |
| maxPacketLossRateDl | O | T | T | F | T |
| maxPacketLossRateUl | O | T | T | F | T |
| extMaxDataBurstVol | O | T | T | F | T |

#### 5.3.84.3	Attribute constraints

None.

#### 5.3.84.4	Notifications

The clause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.85	ARP <<dataType>>

#### 5.3.85.1	Definition

This data type specifies the allocation and retention priority of a QoS control policy.

#### 5.3.85.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| priorityLevel | M | T | T | F | T |
| preemptCap | M | T | T | F | T |
| preemptVuln | M | T | T | F | T |

#### 5.3.85.3	Attribute constraints

None

#### 5.3.85.4	Notifications

The clause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.86	TrafficControlData <<dataType>>

#### 5.3.86.1	Definition

This data type specifies the traffic control data for a service flow of a PCC rule.

#### 5.3.86.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| tcId | M | T | T | F | T |
| flowStatus | M | T | T | F | T |
| redirectInfo | O | T | T | F | T |
| addRedirectInfo | O | T | T | F | T |
| muteNotif | O | T | T | F | T |
| trafficSteeringPolIdDl | O | T | T | F | T |
| trafficSteeringPolIdUl | O | T | T | F | T |
| routeToLocs | M | T | T | F | T |
| traffCorreInd | O | T | T | F | T |
| upPathChgEvent | O | T | T | F | T |
| steerFun | O | T | T | F | T |
| steerModeDl | O | T | T | F | T |
| steerModeUl | O | T | T | F | T |
| mulAccCtrl | O | T | T | F | T |

#### 5.3.86.3	Attribute constraints

None

#### 5.3.86.4	Notifications

The clause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.87	RedirectInformation <<dataType>>

#### 5.3.87.1	Definition

This data type specifies the redirect information for traffic control in the PCC rule.

#### 5.3.87.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| redirectEnabled | M | T | T | F | T |
| redirectAddressType | M | T | T | F | T |
| redirectServerAddress | M | T | T | F | T |

#### 5.3.87.3	Attribute constraints

None

#### 5.3.87.4	Notifications

The clause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.88	RouteToLocation <<dataType>>

#### 5.3.88.1	Definition

This data type specifies a list of location which the traffic shall be routed to for the AF request.

#### 5.3.88.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dnai | M | T | T | F | T |
| routeInfo | CM | T | T | F | T |
| routeProfId | CM | T | T | F | T |

#### 5.3.88.3	Attribute constraints

| Name | Definition |
| --- | --- |
| routeInfo Support Qualifier | Condition: The routeProfId is not supported. |
| routeProfId Support Qualifier | Condition: The routeInfo is not supported. |

#### 5.3.88.4	Notifications

The clause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.89	RouteInformation <<dataType>>

#### 5.3.89.1	Definition

This data type specifies the traffic routing information.

#### 5.3.89.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| ipv4Addr | CM | T | T | F | T |
| ipv6Addr | CM | T | T | F | T |
| portNumber | M | T | T | F | T |

#### 5.3.89.3	Attribute constraints

| Name | Definition |
| --- | --- |
| ipv4Addr Support Qualifier | Condition: The ipv6Addr is not supported. |
| ipv6Addr Support Qualifier | Condition: The ipv4Addr is not supported. |

#### 5.3.89.4	Notifications

The clause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.90	UpPathChgEvent <<dataType>>

#### 5.3.90.1	Definition

This data type specifies the information about the AF subscriptions of the UP path change, see TS 29.512 [60].

#### 5.3.90.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| notificationUri | M | T | T | F | T |
| notifCorreId | M | T | T | F | T |
| dnaiChgType | M | T | T | F | T |
| afAckInd | O | T | T | F | T |

#### 5.3.90.3	Attribute constraints

None

#### 5.3.90.4	Notifications

The clause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.91	SteeringMode <<dataType>>

#### 5.3.91.1	Definition

This data type specifies the traffic distribution rule, see TS 29.512 [60].

#### 5.3.91.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| steerModeValue | M | T | T | F | T |
| active | CM | T | T | F | T |
| standby | O | T | T | F | T |
| threeGLoad | CM | T | T | F | T |
| prioAcc | CM | T | T | F | T |

#### 5.3.91.3	Attribute constraints

| Name | Definition |
| --- | --- |
| active Support Qualifier | Condition: The steerModeValue supports "ACTIVE_STANDBY". |
| threeGLoad Support Qualifier | Condition: The steerModeValue supports "LOAD_BALANCING". |
| prioAcc Support Qualifier | Condition: The steerModeValue supports "PRIORITY_BASED". |

#### 5.3.91.4	Notifications

The clause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.92	ConditionData  <<dataType>>

#### 5.3.92.1	Definition

This data type specifies the condition data for a PCC rule.

#### 5.3.92.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| condId | M | T | T | F | T |
| activationTime | O | T | T | F | T |
| deactivationTime | O | T | T | F | T |
| accessType | O | T | T | F | T |
| ratType | O | T | T | F | T |

#### 5.3.92.3	Attribute constraints

None

#### 5.3.92.4	Notifications

The clause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.93	TscaiInputContainer  <<dataType>>

#### 5.3.93.1	Definition

This data type specifies the transports TSCAI input parameters for TSC traffic at the ingress interface of the DS-TT/UE for a PCC rule, see TS 29.512 [60].

#### 5.3.93.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| periodicity | O | T | T | F | T |
| burstArrivalTime | O | T | T | F | T |

#### 5.3.93.3	Attribute constraints

None

#### 5.3.93.4	Notifications

The clause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.94	Dynamic5QISet

#### 5.3.94.1	Definition

This IOC specifies the dynamic 5QIs including their QoS characteristics, see TS 23.501 [2]. The instance of this IOC shall not be created or modified by the MnS consumer.

#### 5.3.94.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dynamic5QIs | M | T | F | F | T |

#### 5.3.94.3	Attribute constraints

None.

#### 5.3.94.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

## 5.4	Attribute definitions

### 5.4.1	Attribute properties

The following table defines the attributes that are present in several Information Object Classes (IOCs) of the present document.

| Attribute Name |  |  |  | Documentation and Allowed Values |  |  |  | Properties |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| aMFIdentifier |  |  |  | The AMFI is constructed from an AMF Region ID, an AMF Set ID and an AMF Pointer. The AMF Region ID identifies the region, the AMF Set ID uniquely identifies the AMF Set within the AMF Region, and the AMF Pointer uniquely identifies the AMF within the AMF Set. (Ref. TS 23.003 [13]) |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |  |  |
| aMFSetId |  |  |  | It represents the AMF Set ID, which is uniquely identifies the AMF Set within the AMF Region. allowedValues: defined in clause 2.10.1 of TS 23.003 [13]. |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |  |  |
| aMFSetMemberList |  |  |  | It is the list of DNs of AMFFunction instances of the AMFSet. allowedValues: N/A |  |  |  | type: DN multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |  |  |  |
| aMFRegionId |  |  |  | It represents the AMF Region ID, which identifies the region. allowedValues: defined in clause 2.10.1 of TS 23.003 [13]. |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |  |  |
| localAddress |  |  |  | This parameter specifies the localAddress including IP address and VLAN ID used for initialization of the underlying transport. First string is IP address, IP address can be an IPv4 address (See RFC 791 [37]) or an IPv6 address (See RFC 2373 [38]). Second string is VLAN Id (See IEEE 802.1Q [39]). |  |  |  | type: String multiplicity: 2 isOrdered: True isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| remoteAddress |  |  |  | Remote address including IP address used for initialization of the underlying transport. IP address can be an IPv4 address (See RFC 791 [37]) or an IPv6 address (See RFC 2373 [38]). |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| nfProfileList |  |  |  | It is a set of NFProfile(s) to be registered in the NRF instance. NFProfile is defined in TS 29.510 [23]. |  |  |  | type: <<dataType>> multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |  |  |
| cNSIIdList |  |  |  | It is a set of NSI ID. NSI ID is an identifier for identifying the Core Network part of a Network Slice instance when multiple Network Slice instances of the same Network Slice are deployed, and there is a need to differentiate between them in the 5GC, see clause 3.1 of TS 23.501 [2] and clause 6.1.6.2.7 of TS 29.531 [24]. |  |  |  | type: String multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |  |  |
| sNSSAIList |  |  |  | See clause 4.4.1. |  |  |  |  |  |  |  |
| sBIFQDN |  |  |  | It is used to indicate the FQDN of the registered NF instance in service-based interface, for example, NF instance FQDN structure is: nftype<nfnum>.slicetype<sliceid>.mnc<MNC>.mcc<MCC>.3gppnetwork.org |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |  |  |
| sBIServiceList |  |  |  | It is used to indicate the all supported NF services registered on service-based interface. |  |  |  | type: String multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |  |  |
| nRTACList |  |  |  | It is the list of Tracking Area Codes (either legacy TAC or extended TAC). allowedValues: Legacy TAC and Extended TAC are defined in clause 9.3.3.10 of TS 38.413 [5]. |  |  |  | type: Integer multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |  |  |
| supportedBMOList |  |  |  | It is used to indicate the list of supported BMOs (Bridge Managed Objects) required for integration with TSN system. |  |  |  | type: String multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |  |  |
| managedNFProfile |  |  |  | This parameter defines profile for managed NF (See TS 23.501 [22]). allowedValues: N/A |  |  |  | type: ManagedNFProfile multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |  |  |
| nfInstanceID |  |  |  | This parameter defines unique identity of the NF Instance. The format of the NF Instance ID shall be a Universally Unique Identifier (UUID) version 4, as described in IETF RFC 4122 [44] allowedValues: N/A |  |  |  | type: String multiplicity: 1 isOrdered: F isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| nfType |  |  |  | This parameter defines type of Network Function allowedValues: See TS 23.501[22] for NF types |  |  |  | type: ENUM multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| fqdn |  |  |  | This parameter defines FQDN of the Network Function (See TS 23.003 [5]) allowedValues: N/A |  |  |  | type: String multiplicity: 1 isOrdered: F isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| ipAddress |  |  |  | This parameter defines IP Address of the Network Function. It can be IPv4 address (See RFC 791 [37]) or IPv6 address (See RFC 2373 [38]). allowedValues: N/A |  |  |  | type: String multiplicity: 1 isOrdered: F isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| authzInfo |  |  |  | This parameter defines NF Specific Service authorization information. It shall include the NF type (s) and NF realms/origins allowed to consume NF Service(s) of NF Service Producer (See TS 23.501[22]). allowedValues: N/A |  |  |  | type: String multiplicity: 1 isOrdered: F isUnique: N/A defaultValue: None isNullable: True |  |  |  |
| locality |  |  |  | The parameter defines information about the location of the NF instance (e.g. geographic location, data center) defined by operator (See TS 29.510[23]). allowedValues: N/A |  |  |  | type: String multiplicity: 1 isOrdered: F isUnique: N/A defaultValue: None isNullable: True |  |  |  |
| capacity |  |  |  | This parameter defines static capacity information in the range of 0-65535, expressed as a weight relative to other NF instances of the same type; if capacity is also present in the nfServiceList parameters, those will have precedence over this value (See TS 29.510[23]) allowedValues: 0-65535 |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |  |  |
| nFInfo |  |  |  | This parameter includes NF specific data in Managed NF profile allowedValues: N/A |  |  |  | type: NFInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |  |  |
| hostAddr |  |  |  | This parameter defines host address of a NF allowedValues: N/A |  |  |  | type: HostAddr multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |  |  |
| priority |  |  |  | This parameter defines Priority (relative to other NFs of the same type) in the range of 0-65535, to be used for NF selection; lower values indicate a higher priority. If priority is also present in the nfServiceList parameters, those will have precedence over this value (See TS 29.510[23]). allowedValues: 0-65535 |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |  |  |
| supportedDataSetIds |  |  |  | This parameter defines list of supported data sets in the UDR instance (See TS 29.510[23]). allowedValues: "SUBSCRIPTION", "POLICY", EXPOSURE", "APPLICATION" |  |  |  | type: ENUM multiplicity: 1..* isOrdered: N/A isUnique: False defaultValue: None isNullable: False |  |  |  |
| nFSrvGroupId |  |  |  | This parameter defines identity of the group that is served by the NF instance (See TS 29.510[23]). allowedValues: N/A |  |  |  | type: String multiplicity: 1 isOrdered: F isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| smfServingAreas |  |  |  | This parameter defines the SMF service area(s) the UPF can serve (See TS 29.510[23]). allowedValues: N/A |  |  |  | type: String multiplicity: 1..* isOrdered: F isUnique: True defaultValue: None isNullable: False |  |  |  |
| isESCoveredBy |  |  |  | This indicates whether the adjacentCell provides no, partial or full coverage for the cell which name-contains the NRCellRelation instance. Adjacent cells with this attribute equal to "FULL" are recommended to be considered as candidate cells to take over the coverage when the original cell state is about to be changed to energySaving. All adjacent cells with this attribute value equal to "PARTIAL" are recommended to be considered as entirety of candidate cells to take over the coverage when the original cell state is about to be changed to energySaving. allowedValues: NO, PARTIAL, FULL |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| commModelList |  |  |  | The attribute specifies a list of commModel which is defined as a datatype (see clause 5.3.69). It can be used by NF and NF services to interact with each other in 5G Core network (see TS 23.501 [2]). allowedValues: Not applicable |  |  |  | type: commModel multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| groupId |  |  |  | This parameter identiies a list of target NF services on which the same communication model is applied to. allowedValues: N/A |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: False defaultValue: None isNullable: False |  |  |  |
| commModelType |  |  |  | This parameter defines communication model used by a NF to interact with NF service(s) (See TS 23.501 [2]). allowedValues:"DIRECT_COMMUNICATION_WO_NRF", "DIRECT_COMMUNICATION_WITH_NRF", "INDIRECT_COMMUNICATION_WO_DEDICATED_DISCOVERY", "INDIRECT_COMMUNICATION_WITH_DEDICATED_DISCOVERY" |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |  |  |
| targetNFServiceList |  |  |  | This parameter lists target NF services sharing same communication model and configuration. allowedValues: N/A |  |  |  | type: DN multiplicity: 1..* isOrdered: F isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| commModelConfiguration |  |  |  | This parameter defines configuration parameters for specific communication model for a group of NF Services. allowedValues: N/A |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |  |  |
| supportedFuncList |  |  |  | This parameter lists functionalities supported by a SCP. Refer to TS 23.501 [2]. |  |  |  | type: SupportedFunction multiplicity: 1..* isOrdered: N/A isUnique: False defaultValue: None isNullable: False |  |  |  |
| address |  |  |  | This parameter defines address of a SCP instance, it can be IP address (either IPv4 address (See RFC 791 [37]) or IPv6 address (See RFC 2373 [38])) or FQDN (See TS 23.003 [5]). |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |  |  |
| function |  |  |  | This parameter defines name of a functionality supported by a SCP. |  |  |  | type: String multiplicity: 1 isOrdered: F isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| policy |  |  |  | This parameter defines configuration policies of a functionality supported by a SCP. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |  |  |
| capabilityList |  |  |  | This parameter lists capabilities supported by a NEF. Refer to TS 23.501 [2]. allowedValues: N/A |  |  |  | type: String multiplicity: 1..* isOrdered: N/A isUnique: False defaultValue: None isNullable: False |  |  |  |
| isINEF |  |  |  | This parameter defines if the NEF is an Intermediate NEF. allowedValues: TRUE, FALSE |  |  |  | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |  |  |
| isCAPIFSup |  |  |  | This parameter defines if the NEF support Common API Framework. allowedValues: TRUE, FALSE |  |  |  | type: Boolean multiplicity: 1 isOrdered: F isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| sEPPType |  |  |  | This parameter defines the type of a SEPP entity. Refer to TS 33.501 [52]. allowedValues: "CSEPP", "PSEPP" |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: False defaultValue: None isNullable: False |  |  |  |
| sEPPId |  |  |  | This parameter is identifier of a SEPP, it is unique inside a PLMN. allowedValues: N/A |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |  |  |
| remotePlmnId |  |  |  | This parameter defines PLMNId of the remote SEPP. allowedValues: N/A |  |  |  | Type: PLMNId multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| remoteSeppAddress |  |  |  | This parameter defines address of the remote SEPP. It can be IP address (either IPv4 address (See RFC 791 [37]) or IPv6 address (See RFC 2373 [38])) or FQDN(See TS 23.003 [5]). allowedValues: N/A |  |  |  | type: String multiplicity: 1 isOrdered: F isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| remoteSeppId |  |  |  | This parameter defines identifier of the remote SEPP. it is unique inside a PLMN. allowedValues: N/A |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |  |  |
| n32cParas |  |  |  | This attribute is used to configure parameters to establish security link between two SEPPs. allowedValues: N/A |  |  |  | type: String multiplicity: 1 isOrdered: F isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| n32fPolicy |  |  |  | This attribute is used to configure policies to protect the messages exchanged between SEPPs. allowedValues: N/A |  |  |  | type: String multiplicity: 1 isOrdered: F isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| withIPX |  |  |  | This attribute defines if there's an IPX interconnected between two SEPPs. allowedValues: TRUE, FALSE |  |  |  | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |  |  |
| FiveQiDscpMappingList |  |  |  | It provides the list of mapping between 5QIs and DSCP. allowedValues: N/A |  |  |  | type: FiveQiDscpMapping multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| fiveQIValues |  |  |  | It indicates a list of 5QI value. allowedValues: 0 - 255 |  |  |  | type: Integer multiplicity: * isOrdered: N/A isUnique: Yes defaultValue: None isNullable: False |  |  |  |
| dscp |  |  |  | It indicates a DSCP. allowedValues: 0 - 255 |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: Yes defaultValue: None isNullable: False |  |  |  |
| configurable5QISetRef |  |  |  | This is the DN of Configurable5QISet. allowedValues: DN of the Configurable5QISet MOI. |  |  |  | type: String multiplicity: 0..1 isOrdered: False isUnique: True defaultValue: None isNullable: True |  |  |  |
| configurable5QIs |  |  |  | It indicates the configurable 5QIs, including their QoS characteristics. allowedValues: N/A |  |  |  | type: FiveQICharacteristics multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| dynamic5QISetRef |  |  |  | This is the DN of Dynamic5QISet MOI. allowedValues: DN of the Dynamic5QISet MOI. |  |  |  | type: String multiplicity: 0..1 isOrdered: False isUnique: True defaultValue: None isNullable: True |  |  |  |
| dynamic5QIs |  |  |  | It indicates the dynamic 5QIs, including their QoS characteristics. allowedValues: N/A |  |  |  | type: FiveQICharacteristics multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| fiveQIValue |  |  |  | It identifies the 5QI value. allowedValues: 0 - 255 |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: Yes defaultValue: None isNullable: False |  |  |  |
| resourceType |  |  |  | It indicates the Resource Type of a 5QI, as specified in TS 23.501 [2]. allowedValues: "GBR", "Non-GBR" |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: False defaultValue: None isNullable: False |  |  |  |
| priorityLevel |  |  |  | It indicates the Priority Level of a 5QI, as specified in TS 23.501 [2]. allowedValues: 0 - 127 |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: False defaultValue: None isNullable: False |  |  |  |
| packetDelayBudget |  |  |  | It indicates the Packet Delay Budget (in unit of 0.5ms) of a 5QI, as specified in TS 23.501 [2]. allowedValues: 0 - 1023 |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: False defaultValue: None isNullable: False |  |  |  |
| packetErrorRate |  |  |  | It indicates the Packet Error Rate of a 5QI, as specified in TS 23.501 [2]. allowedValues: N/A |  |  |  | type: PacketErrorRate multiplicity: 1 isOrdered: N/A isUnique: False defaultValue: None isNullable: False |  |  |  |
| averagingWindow |  |  |  | It indicates the Averaging Window (in unit of ms) of a 5QI, as specified in TS 23.501 [2]. allowedValues: 0 - 4095 |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: False defaultValue: None isNullable: False |  |  |  |
| maximumDataBurstVolume |  |  |  | It indicates the Maximum Data Burst Volume (in unit of Byte) of a 5QI, as specified in TS 23.501 [2]. allowedValues: 0 - 4095 |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: False defaultValue: None isNullable: False |  |  |  |
| scalar |  |  |  | The Packet Error Rate of a 5QI expressed as Scalar x 10-k where k is the Exponent. This attriutes indicates the Scalar of this expression. allowedValues: 0 - 9 |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: False defaultValue: None isNullable: False |  |  |  |
| exponent |  |  |  | The Packet Error Rate of a 5QI expressed as Scalar x 10-k where k is the Exponent. This attriutes indicates the Exponent of this expression. allowedValues: 0 - 9 |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: False defaultValue: None isNullable: False |  |  |  |
| gtpUPathQoSMonitoringState |  |  |  | It indicates the state of GTP-U path QoS monitoring for URLLC service. allowedValues: "Enabled", "Disabled". |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Enabled isNullable: False |  |  |  |
| gtpUPathMonitoredSNSSAIs |  |  |  | It specifies the S-NSSAIs for which the GTP-U path QoS monitoring is to be performed. allowedValues: See TS 23.003 [13] |  |  |  | type: S-NSSAI multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| monitoredDSCPs |  |  |  | It specifies the DSCPs for which the GTP-U path QoS monitoring is to be performed. allowedValues: See TS 29.244 [56] |  |  |  | type: Integer multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| isEventTriggeredGtpUPathMonitoringSupported |  |  |  | It indicates whether the event triggered GTP-U path QoS monitoring reporting based on thresholds is supported, see TS 29.244 [56]. allowedValues: "Yes", "No". |  |  |  | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Yes isNullable: False |  |  |  |
| isPeriodicGtpUMonitoringSupported |  |  |  | It indicates whether the periodic GTP-U path QoS monitoring reporting is supported, see TS 29.244 [56]. allowedValues: "Yes", "No". |  |  |  | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Yes isNullable: False |  |  |  |
| isImmediateGtpUMonitoringSupported |  |  |  | It indicates whether the immediate GTP-U path QoS monitoring reporting is supported, see TS 29.244 [56]. allowedValues: "Yes", "No". |  |  |  | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Yes isNullable: False |  |  |  |
| gtpUPathDelayThresholds |  |  |  | It specifies the thresholds for reporting the packet delay for the GTO-U path QoS monitoring, if the isEventTriggeredGtpUPathMonitoringSupported attribute of the same MOI is set to "yes". The packet delay will be reported to SMF when it exceeds the threshold (in milliseconds). allowedValues: N/A. |  |  |  | type: GtpUPathDelayThresholdsType multiplicity: 1 isOrdered: Y isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| gtpUPathMinimumWaitTime |  |  |  | It specifies the minimum waiting time (in seconds) between two consecutive reports for event triggered GTP-U path QoS monitoring reporting, if the isEventTriggeredGtpUPathMonitoringSupported attribute of the same MOI is set to "yes". allowedValues: see TS 29.244 [56]. |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| gtpUPathMeasurementPeriod |  |  |  | It specifies the period (in seconds) for reporting the packet delay for GTP-U path QoS monitoring, if the isPeriodicGtpUMonitoringSupported attribute of the same MOI is set to "yes". allowedValues: see TS 29.244 [56]. |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| n3AveragePacketDelayThreshold |  |  |  | It specifies the threshold for reporting the average packet delay of a GTP-U path on N3 interface. allowedValues: see TS 29.244 [56]. |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| n3MinPacketDelayThreshold |  |  |  | It specifies the threshold for reporting the minimum packet delay of a GTP-U path on N3 interface. allowedValues: see TS 29.244 [56]. |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| n3MaxPacketDelayThreshold |  |  |  | It specifies the threshold for reporting the maxinum packet delay of a GTP-U path on N3 interface. allowedValues: see TS 29.244 [56]. |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| n9AveragePacketDelayThreshold |  |  |  | It specifies the threshold for reporting the average packet delay of a GTP-U path on N9 interface. allowedValues: see TS 29.244 [56]. |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| n9MinPacketDelayThreshold |  |  |  | It specifies the threshold for reporting the minimum packet delay of a GTP-U path on N9 interface. allowedValues: see TS 29.244 [56]. |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| n9MaxPacketDelayThreshold |  |  |  | It specifies the threshold for reporting the maxinum packet delay of a GTP-U path on N9 interface. allowedValues: see TS 29.244 [56]. |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| qFQoSMonitoringState |  |  |  | It indicates the state of QoS monitoring per QoS flow per UE for URLLC service. allowedValues: "Enabled", "Disabled". |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Enabled isNullable: False |  |  |  |
| qFMonitoredSNSSAIs |  |  |  | It specifies the S-NSSAIs for which the QoS monitoring per QoS flow per UE is to be performed. allowedValues: See TS 23.003 [13] |  |  |  | type: S-NSSAI multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| qFMonitored5QIs |  |  |  | It specifies the 5QIs for which the QoS monitoring per QoS flow per UE is to be performed. allowedValues: See TS 23.501[2] |  |  |  | type: Integer multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| isEventTriggeredQFMonitoringSupported |  |  |  | It indicates whether the event based QoS monitoring reporting per QoS flow per UE is supported, see TS 29.244 [56]. allowedValues: "Yes", "No". |  |  |  | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Yes isNullable: False |  |  |  |
| isPeriodicQFMonitoringSupported |  |  |  | It indicates whether the periodic QoS monitoring reporting per QoS flow per UE is supported, see TS 29.244 [56]. allowedValues: "Yes", "No". |  |  |  | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Yes isNullable: False |  |  |  |
| isSessionReleasedQFMonitoringSupported |  |  |  | It indicates whether the session release based QoS monitoring reporting per QoS flow per UE is supported, see TS 29.244 [56]. allowedValues: "Yes", "No". |  |  |  | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Yes isNullable: False |  |  |  |
| qFPacketDelayThresholds |  |  |  | It specifies the thresholds for reporting the packet delay between PSA and UE for QoS monitoring per QoS flow per UE, if the isEventTriggeredQFMonitoringSupported attribute of the same MOI is set to "yes".". The packet delay will be reported by PSA UPF to SMF when it exceeds the threshold (in milliseconds). allowedValues: see TS 29.244 [56]. |  |  |  | type: QFPacketDelayThresholdsType multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| qFMinimumWaitTime |  |  |  | It specifies the minimum waiting time (in seconds) between two consecutive reports for event triggered QoS monitoring reporting per QoS flow per UE, if the isEventTriggeredQFMonitoringSupported attribute of the same MOI is set to "yes". allowedValues: see TS 29.244 [56]. |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| qFMeasurementPeriod |  |  |  | It specifies the period (in seconds) for reporting the packet delay for QoS monitoring per QoS flow per UE, if the isPeriodicQFMonitoringSupported attribute of the same MOI is set to "yes". allowedValues: see TS 29.244 [56]. |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| thresholdDl |  |  |  | It specifies the threshold for reporting the DL packet delay between PSA UPF and UE. allowedValues: see TS 29.244 [56]. |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| thresholdUl |  |  |  | It specifies the threshold for reporting the UL packet delay between PSA UPF and UE. allowedValues: see TS 29.244 [56]. |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| thresholdRtt |  |  |  | It specifies the threshold for reporting the round-trip packet delay between PSA UPF and UE. allowedValues: see TS 29.244 [56]. |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| predefinedPccRules |  |  |  | It specifies the predefined PCC Rules, see TS 25.503 [59]. allowedValues: N/A |  |  |  | type: PccRule multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| pccRuleId |  |  |  | It identifies the PCC rule. allowedValues: N/A |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| flowInfoList |  |  |  | It is a list of IP flow packet filter information. allowedValues: N/A |  |  |  | type: FlowInformation multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| applicationId |  |  |  | A reference to the application detection filter configured at the UPF. allowedValues: N/A |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| appDescriptor |  |  |  | It is the ATSSS rule application descriptor. allowedValues: see TS 29.571 [61]. |  |  |  | type: BitString multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| contentVersion |  |  |  | Indicates the content version of the PCC rule. allowedValues: N/A |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| precedence |  |  |  | It indicates the order in which this PCC rule is applied relative to other PCC rules within the same PDU session. allowedValues: 0..255. |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| afSigProtocol |  |  |  | Indicates the protocol used for signalling between the UE and the AF. The default value is "NO_INFORMATION". allowedValues: "NO_INFORMATION", "SIP". |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "NO_INFORMATION" isNullable: False |  |  |  |
| isAppRelocatable |  |  |  | It indicates the application relocation possibility. The default value is "FALSE. allowedValues: "TRUE", "FALSE". |  |  |  | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| isUeAddrPreserved |  |  |  | It Indicates whether UE IP address should be preserved. The default value is "FALSE". allowedValues: "TRUE", "FALSE". |  |  |  | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "FALSE" isNullable: False |  |  |  |
| qosData |  |  |  | It contains the QoS control policy data for a PCC rule. allowedValues: N/A |  |  |  | type: QoSData multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| altQosParams |  |  |  | It contains the QoS control policy data for the Alternative QoS parameter sets of the service data flow. Only the "qosId" attribute, "5qi" attribute, "maxbrUl" attribute, "maxbrDl" attribute, "gbrUl" attribute and "gbrDl" attribute are applicable within the QosData data type. This data type represents an ordered list, where the lower the index of the array for a given entry, the higher the priority. allowedValues: N/A |  |  |  | type: QoSData multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| trafficControlData |  |  |  | It contains the traffic control policy data for a PCC rule. allowedValues: N/A |  |  |  | type: TrafficControlData multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| conditionData |  |  |  | It contains the condition data for a PCC rule. allowedValues: N/A |  |  |  | type: ConditionData multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| tscaiInputUl |  |  |  | It contains transports TSCAI input parameters for TSC traffic at the ingress interface of the DS-TT/UE (uplink flow direction). allowedValues: N/A |  |  |  | type: TscaiInputContainer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| tscaiInputDl |  |  |  | It contains transports TSCAI input parameters for TSC traffic at the ingress of the NW-TT (downlink flow direction). allowedValues: N/A |  |  |  | type: TscaiInputContainer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| flowDescription |  |  |  | It defines a packet filter for an IP flow. allowedValues: see TS 29.214 [62]. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| ethFlowDescription |  |  |  | It defines a packet filter for an Ethernet flow. allowedValues: see TS 29.514 [62]. |  |  |  | type: EthFlowDescription multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| destMacAddr |  |  |  | It specifies the destination MAC address formatted in the hexadecimal notation according to clause 1.1 and clause 2.1 of IETF RFC 7042 [63]. Pattern: '^([0-9a-fA-F]{2})((-[0-9a-fA-F]{2}){5})$'. allowedValues: N/A. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| ethType |  |  |  | A two-octet string that represents the Ethertype, as described in IEEE 802.3 [64] and IETF RFC 7042 [63] in hexadecimal representation. Each character in the string shall take a value of "0" to "9" or "A" to "F" and shall represent 4 bits. The most significant character representing the 4 most significant bits of the ethType shall appear first in the string, and the character representing the 4 least significant bits of the ethType shall appear last in the string. allowedValues: see IEEE 802.3 [64] and IETF RFC 7042 [63]. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| fDesc |  |  |  | It contains the flow description for the Uplink or Downlink IP flow. It shall be present when the ethtype is IP. allowedValues: see flowDescription in TS 29.214 [62]. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| fDir |  |  |  | It indicates the packet filter direction. allowedValues: "DOWNLINK", "UPLINK". |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| sourceMacAddr |  |  |  | It specifies the source MAC address formatted in the hexadecimal notation according to clause 1.1 and clause 2.1 of IETF RFC 7042 [63]. Pattern: '^([0-9a-fA-F]{2})((-[0-9a-fA-F]{2}){5})$'. allowedValues: N/A. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| vlanTags |  |  |  | It specifies the Customer-VLAN and/or Service-VLAN tags containing the VID, PCP/DEI fields as defined in IEEE 802.1Q [65] and IETF RFC 7042 [63]. The first/lower instance in the array stands for the Customer-VLAN tag and the second/higher instance in the array stands for the Service-VLAN tag. Each field is encoded as a two-octet string in hexadecimal representation. Each character in the string shall take a value of "0" to "9" or "A" to "F" and shall represent 4 bits. The most significant character representing the PCP/DEI field shall appear first in the string, followed by character representing the 4 most significant bits of the VID field, and the character representing the 4 least significant bits of the VID field shall appear last in the string. If only Service-VLAN tag is provided, empty string for Customer-VLAN tag shall be provided. allowedValues: see IEEE 802.1Q [65] and IETF RFC 7042 [63]. |  |  |  | type: String multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| srcMacAddrEnd |  |  |  | It specifies the source MAC address end. If this attribute is present, the sourceMacAddr attribute specifies the source MAC address start. E.g. srcMacAddrEnd with value 00-10-A4-23-3E-FE and sourceMacAddr with value 00-10-A4-23-3E-02 means all MAC addresses from 00-10-A4-23-3E-02 up to and including 00-10-A4-23-3E-FE. allowedValues: N/A. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |  |  |  |
| destMacAddrEnd |  |  |  | It specifies the destination MAC address end. If this attribute is present, the destMacAddr attribute specifies the destination MAC address start. allowedValues: N/A. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |  |  |  |
| packFiltId |  |  |  | It is the identifier of the packet filter. allowedValues: N/A. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| packetFilterUsage |  |  |  | It indicates if the packet shall be sent to the UE. The default value is "FALSE". allowedValues: TRUE, FALSE |  |  |  | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "FALSE" isNullable: False |  |  |  |
| tosTrafficClass |  |  |  | It contains the Ipv4 Type-of-Service and mask field or the Ipv6 Traffic-Class field and mask field. allowedValues: N/A |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| spi |  |  |  | It is the security parameter index of the IPSec packet, see IETF RFC 4301 [66]. allowedValues: see IETF RFC 4301 [66]. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |  |  |  |
| flowLabel |  |  |  | It specifies the Ipv6 flow label header field. AllowedValues: N/A |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |  |  |  |
| flowDirection |  |  |  | It indicates the direction/directions that a filter is applicable. AllowedValues: "DOWNLINK", "UPLINK", "BIDIRECTIONAL", "UNSPECIFIED". |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |  |  |  |
| qosId |  |  |  | It identifies the QoS control policy data for a PCC rule. AllowedValues: N/A |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| maxbrUl |  |  |  | It represents the maximum uplink bandwidth formatted as follows: Pattern: '^\d+(\.\d+)? (bps¦Kbps¦Mbps¦Gbps¦Tbps)$', see TS 29.512 [60]. Examples: "125 Mbps", "0.125 Gbps", "125000 Kbps" AllowedValues: N/A |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |  |  |  |
| maxbrDl |  |  |  | It represents the maximum downlink bandwidth formatted as follows: Pattern: '^\d+(\.\d+)? (bps¦Kbps¦Mbps¦Gbps¦Tbps)$', see TS 29.512 [60]. Examples: "125 Mbps", "0.125 Gbps", "125000 Kbps". AllowedValues: N/A. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |  |  |  |
| gbrUl |  |  |  | It represents the guaranteed uplink bandwidth formatted as follows: Pattern: '^\d+(\.\d+)? (bps¦Kbps¦Mbps¦Gbps¦Tbps)$', see TS 29.512 [60]. Examples: "125 Mbps", "0.125 Gbps", "125000 Kbps". AllowedValues: N/A. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |  |  |  |
| gbrDl |  |  |  | It represents the guaranteed downlink bandwidth formatted as follows: Pattern: '^\d+(\.\d+)? (bps¦Kbps¦Mbps¦Gbps¦Tbps)$', see TS 29.512 [60]. Examples: "125 Mbps", "0.125 Gbps", "125000 Kbps". AllowedValues: N/A. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |  |  |  |
| extMaxDataBurstVol |  |  |  | It denotes the largest amount of data that is required to be transferred within a period of 5G-AN PDB, see TS 29.512 [60]. AllowedValues: 4096..2000000. |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |  |  |  |
| arp |  |  |  | It indicates the allocation and retention priority. AllowedValues: N/A. |  |  |  | type: ARP multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| ARP.priorityLevel |  |  |  | It defines the relative importance of a resource request. AllowedValues: 1..15. |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| preemptCap |  |  |  | It defines whether a service data flow may get resources that were already assigned to another service data flow with a lower priority level. AllowedValues: "NOT_PREEMPT", "MAY_PREEMPT". |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| preemptVuln |  |  |  | It defines whether a service data flow may lose the resources assigned to it in order to admit a service data flow with higher priority level. AllowedValues: "NOT_PREEMPTABLE", "PREEMPTABLE". |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| qosNotificationControl |  |  |  | It indicates whether notifications are requested from 3GPP NG-RAN when the GFBR can no longer (or again) be guaranteed for a QoS Flow during the lifetime of the QoS Flow. The default value is "FALSE". AllowedValues: "TRUE", "FALSE". |  |  |  | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "FALSE" isNullable: False |  |  |  |
| reflectiveQos |  |  |  | Indicates whether the QoS information is reflective for the corresponding non-GBR service data flow. The default value is "FALSE". AllowedValues: "TRUE", "FALSE". |  |  |  | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "FALSE" isNullable: False |  |  |  |
| sharingKeyDl |  |  |  | It indicates, by containing the same value, what PCC rules may share resource in downlink direction. AllowedValues: N/A. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |  |  |  |
| sharingKeyUl |  |  |  | It indicates, by containing the same value, what PCC rules may share resource in uplink direction. AllowedValues: N/A. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |  |  |  |
| maxPacketLossRateDl |  |  |  | It indicates the downlink maximum rate for lost packets that can be tolerated for the service data flow. AllowedValues: 0..1000. |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |  |  |  |
| maxPacketLossRateUl |  |  |  | It indicates the uplink maximum rate for lost packets that can be tolerated for the service data flow. AllowedValues: 0..1000. |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |  |  |  |
| tcId |  |  |  | It univocally identifies the traffic control policy data within a PDU session. AllowedValues: N/A. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| flowStatus |  |  |  | It represents whether the service data flow(s) are enabled or disabled. The default value is "ENABLED". See TS 29.514 [67]. AllowedValues: "ENABLED-UPLINK", "ENABLED-DOWNLINK", "ENABLED", "DISABLED", "REMOVED". |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "ENABLED" isNullable: False |  |  |  |
| redirectInfo |  |  |  | It indicates whether the detected application traffic should be redirected to another controlled address. AllowedValues: N/A. |  |  |  | type: RedirectInformation multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "ENABLED" isNullable: False |  |  |  |
| addRedirectInfo |  |  |  | It contains the additional redirect information indicating whether the detected application traffic should be redirected to another controlled address. AllowedValues: N/A. |  |  |  | type: RedirectInformation multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: "ENABLED" isNullable: False |  |  |  |
| redirectEnabled |  |  |  | It indicates whether the redirect instruction is enabled. AllowedValues: "TRUE", "FALSE". |  |  |  | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| redirectAddressType |  |  |  | It indicates the type of redirect address, see TS 29.512 [60]. AllowedValues: " IPV4_ADDR", "IPV6_ADDR", "URL", "SIP_URI". |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| redirectServerAddress |  |  |  | It indicates the address of the redirect server. AllowedValues: N/A. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| muteNotif |  |  |  | It indicates whether applicat'on's start or stop notification is to be muted. The default value is "FALSE". AllowedValues: "TRUE", "FALSE". |  |  |  | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "FALSE" isNullable: False |  |  |  |
| trafficSteeringPolIdDl |  |  |  | It references to a pre-configured traffic steering policy for downlink traffic at the SMF, see TS 29.512 [60]. AllowedValues: N/A. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| trafficSteeringPolIdUl |  |  |  | It references to a pre-configured traffic steering policy for uplink traffic at the SMF, see TS 29.512 [60]. AllowedValues: N/A. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| routeToLocs |  |  |  | It provides a list of location which the traffic shall be routed to for the AF request. AllowedValues: N/A. |  |  |  | type: RouteToLocation multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| traffCorreInd |  |  |  | It indicates the traffic correlation. AllowedValues: "TRUE", "FALSE". |  |  |  | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "FALSE" isNullable: False |  |  |  |
| dnai |  |  |  | It represents the DNAI (Data network access identifier), see TS 23.501 [2]. AllowedValues: N/A. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| routeInfo |  |  |  | It provides the traffic routing information. AllowedValues: N/A. |  |  |  | type: RouteInformation multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| ipv4Addr |  |  |  | It defines the Ipv4 address of the tunnel end point in the data network, formatted in the "dotted decimal" notation. Pattern: '^(([0-9]¦[1-9][0-9]¦1[0-9][0-9]¦2[0-4][0-9]¦25[0-5])\.){3}([0-9]¦[1-9][0-9]¦1[0-9][0-9]¦2[0-4][0-9]¦25[0-5])$'. AllowedValues: N/A. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| ipv6Addr |  |  |  | It defines the Ipv6 address of the tunnel end point in the data network. Pattern: '^((:¦(0?¦([1-9a-f][0-9a-f]{0,3}))):)((0?¦([1-9a-f][0-9a-f]{0,3})):){0,6}(:¦(0?¦([1-9a-f][0-9a-f]{0,3})))$' and Pattern: '^((([^:]+:){7}([^:]+))¦((([^:]+:)*[^:]+)?::(([^:]+:)*[^:]+)?))$'. AllowedValues: N/A. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| portNumber |  |  |  | It defines the UDP port number of the tunnel end point in the data network, see TS 29.571 [61]. AllowedValues: N/A. |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| routeProfId |  |  |  | It identifies the routing profile. AllowedValues: N/A. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| upPathChgEvent |  |  |  | It contains the information about the AF subscriptions of the UP path change. AllowedValues: N/A. |  |  |  | type: UpPathChgEvent multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| notificationUri |  |  |  | It provides notification address (Uri) of AF receiving the event notification. AllowedValues: N/A. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| notifCorreId |  |  |  | It is used to set the value of Notification Correlation ID in the notification sent by the SMF, see TS 29.512 [60]. AllowedValues: N/A. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| dnaiChgType |  |  |  | It indicates the type of DNAI change, see TS 29.512 [60]. AllowedValues: "EARLY", "EARLY_LATE", "LATE". |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| afAckInd |  |  |  | It identifies whether the AF acknowledgement of UP path event notification is expected.The default value is "FALSE". AllowedValues: "TRUE", "FALSE". |  |  |  | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "FALSE" isNullable: False |  |  |  |
| steerFun |  |  |  | It indicates the applicable traffic steering functionality, see TS 29.512 [60]. AllowedValues: "MPTCP", "ATSSS_LL". |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| steerModeDl |  |  |  | It provides the traffic distribution rule across 3GPP and Non-3GPP accesses to apply for downlink traffic. AllowedValues: N/A. |  |  |  | type: SteeringMode multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| steerModeUl |  |  |  | It provides the traffic distribution rule across 3GPP and Non-3GPP accesses to apply for uplink traffic. AllowedValues: N/A. |  |  |  | type: SteeringMode multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| mulAccCtrl |  |  |  | It indicates whether the service data flow, corresponding to the service data flow template, is allowed or not allowed. The default value is "NOT_ALLOWED". AllowedValues: "ALLOWED", "NOT_ALLOWED". |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "NOT_ALLOWED" isNullable: False |  |  |  |
| steerModeValue |  |  |  | It indicates the value of the steering mode, see TS 29.512 [60]. AllowedValues: "ACTIVE_STANDBY", "LOAD_BALANCING", "SMALLEST_DELAY", "PRIORITY_BASED". |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| active |  |  |  | It indicates the active access, see TS 29.571 [61]. AllowedValues: "3GPP_ACCESS", "NON_3GPP_ACCESS". |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| standby |  |  |  | It indicates the Standby access, see TS 29.571 [61]. AllowedValues: "3GPP_ACCESS", "NON_3GPP_ACCESS". |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| threeGLoad |  |  |  | It indicates the traffic load to steer to the 3GPP Access expressed in one percent. AllowedValues: 0..100. |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| prioAcc |  |  |  | It indicates the high priority access, see TS 29.571 [61]. AllowedValues: "3GPP_ACCESS", "NON_3GPP_ACCESS". |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| condId |  |  |  | It uniquely identifies the condition data. AllowedValues: N/A. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| activationTime |  |  |  | It indicates the time (in date-time format) when the decision data shall be activated, see TS 29.512 [60] and TS 29.571 [61]. AllowedValues: N/A. |  |  |  | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| deactivationTime |  |  |  | It indicates the time (in date-time format) when the decision data shall be deactivated, see TS 29.512 [60] and TS 29.571 [61]. AllowedValues: N/A. |  |  |  | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| accessType |  |  |  | It provides the condition of access type of the UE when the session AMBR shall be enforced, see TS 29.512 [60]. AllowedValues: "3GPP_ACCESS", "NON_3GPP_ACCESS". |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| ratType |  |  |  | It provides the condition of RAT type of the UE when the session AMBR shall be enforced, see TS 29.512 [60] and TS 29.571 [61]. AllowedValues: "NR", "EUTRA", "WLAN", "VIRTUAL", "NBIOT", "WIRELINE", "WIRELINE_CABLE", "WIRELINE_BBF", "LTE-M", "NR_U", "EUTRA_U", "TRUSTED_N3GA", "TRUSTED_WLAN", "UTRA", "GERA". |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| periodicity |  |  |  | It identifies the time period between the start of two bursts in reference to the TSN GM. AllowedValues: see TS 29.571 [61]. |  |  |  | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| burstArrivalTime |  |  |  | Indicates the arrival time (in date-time format) of the data burst in reference to the TSN GM. AllowedValues: see TS 29.571 [61]. |  |  |  | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |

## 5.5	Common notifications

### 5.5.1	Alarm notifications

This clause presents a list of notifications, defined in TS 28.532 [35], that an MnS consumer may receive. The notification header attribute objectClass/objectInstance shall capture the DN of an instance of a class defined in the present document.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyNewAlarm | M | -- |
| notifyClearedAlarm | M | -- |
| notifyAckStateChanged | M | -- |
| notifyAlarmListRebuilt | M | -- |
| notifyChangedAlarm | O | -- |
| notifyCorrelatedNotificationChanged | O | -- |
| notifyChangedAlarmGeneral | O | -- |
| notifyComments | O | -- |
| notifyPotentialFaultyAlarmList | O | -- |

### 5.5.2	Configuration notifications

This clause presents a list of notifications, defined in TS 28.532 [35], that an MnS consumer may receive. The notification header attribute objectClass/objectInstance shall capture the DN of an instance of a class defined in the present document.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyMOICreation | O | -- |
| notifyMOIDeletion | O | -- |
| notifyMOIAttributeValueChanges | O | -- |
| notifyEvent | O | -- |

# 6	Information model definitions for network slice NRM

## 6.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.622 [30], IOC, Top | Top |
| TS 28.622 [30], IOC, SubNetwork | SubNetwork |
| TS 28.622 [30], IOC, ManagedFunction | ManagedFunction |
| TS 28.658 [19], dataType, PLMNId | PLMNId |

## 6.2	Class diagram

### 6.2.1	Relationships

Figure 6.2.1-1: Network slice NRM fragment relationship

NOTE 1:	The <<OpenModelClass>> NetworkService and <<OpenModelClass>> VNF are defined in [40].

NOTE 2:	The target Network Service (NS) instance represents a group of VNFs and PNFs that are supporting the source network slice subnet instance.

NOTE 3:	The instance tree of this NRM fragment would not contain the instances of NetworkService and VNF. However, the NetworkSliceSubNet instances would have an attribute holding the identifiers of NetworkService instances and the ManagedFunction instance would have an attribute holding identifiers of VNF instances.

Figure 6.2.1-2: Transport EP NRM fragment relationship

### 6.2.2	Inheritance

Figure 6.2.2-1: Network slice inheritance relationship

## 6.3	Class definitions

### 6.3.1	NetworkSlice

#### 6.3.1.1	Definition

This IOC represents the properties of a network slice instance in a 5G network. For more information about the network slice instance, see TS 28.531 [26].

#### 6.3.1.2	Attributes

The NetworkSlice IOC includes attributes inherited from SubNetwork IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |  |
| --- | --- | --- | --- | --- | --- | --- |
| operationalState | M | T | F | F | T |  |
| administrativeState | M | T | T | F | T |
| serviceProfileList | M | T | T | F | T |  |
| Attribute related to role |  |  |  |  |  |  |
| networkSliceSubnetRef | M | T | F | F | T |  |

#### 6.3.1.3	Attribute constraints

None.

#### 6.3.1.4	Notifications

The common notifications defined in clause 6.5 are valid for this IOC, without exceptions or additions.

### 6.3.2	NetworkSliceSubnet

#### 6.3.2.1	Definition

This IOC represents the properties of a network slice subnet instance in a 5G network. For more information about the network slice subnet instance, see TS 28.531 [26].

#### 6.3.2.2	Attributes

The NetworkSliceSubnet IOC includes attributes inherited from SubNetwork IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| operationalState | M | T | F | F | T |
| administrativeState | M | T | T | F | T |
| nsInfo | CM | T | F | F | T |
| sliceProfileList | M | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| managedFunctionRef | M | T | F | F | T |
| networkSliceSubnetRef | M | T | F | F | T |

#### 6.3.2.3	Attribute constraints

| Name | Definition |
| --- | --- |
| nsInfo Support Qualifier | Condition: It shall be supported if the NSS instance is realized in the virtualized environment. Otherwise this attribute shall be absent. |

#### 6.3.2.4	Notifications

The common notifications defined in clause 6.5 are valid for this IOC, without exceptions or additions.

### 6.3.3	ServiceProfile <<dataType>>

#### 6.3.3.1	Definition

This data type represents the properties of network slice related requirement that should be supported by the network slice instance in 5G network. The network slice can be tailored based on the specific requirements adhered to SLA agreed between Network Slice Customer (NSC) and Network Slice Provider (NSP), see clause 2 of [50]. A network slicing provider may add additional requirements not directly derived from SLA's, associated to the provider internal [business] goals. The GST defined by GSMA (see [50]) and the service performance requirements defined in TS 22.261 [28] and TS 22.104 [51] are all considered as input for the network slice related requirements.

#### 6.3.3.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| serviceProfileId | M | T | F | T | T |
| sNSSAIList | M | T | T | F | T |
| pLMNIdList | M | T | T | F | T |
| maxNumberofUEs | O | T | T | F | T |
| coverageArea | O | T | T | F | T |
| latency | O | T | T | F | T |
| uEMobilityLevel | O | T | T | F | T |
| resourceSharingLevel | O | T | T | F | T |
| sST | M | T | T | F | T |
| availability | O | T | T | F | T |
| delayTolerance | O | T | T | F | T |
| deterministicComm | O | T | T | F | T |
| dLThptPerSlice | O | T | T | F | T |
| dLThptPerUE | O | T | T | F | T |
| uLThptPerSlic | O | T | T | F | T |
| uLThptPerUE | O | T | T | F | T |
| maxPktSize | O | T | T | F | T |
| maxNumberofConns | O | T | T | F | T |
| kPIMonitoring | O | T | T | F | T |
| userMgmtOpen | O | T | T | F | T |
| v2XCommModels | O | T | T | F | T |
| termDensity | O | T | T | F | T |
| activityFactor | O | T | T | F | T |
| uESpeed | O | T | T | F | T |
| jitter | O | T | T | F | T |
| survivalTime | O | T | T | F | T |
| reliability | O | T | T | F | T |

NOTE:	The attributes in ServiceProfile represent mapped requirements from an NSC (e.g. an enterprise) to an NSP

#### 6.3.3.3	Attribute constraints

None.

#### 6.3.3.4	Notifications

The clause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.4	SliceProfile <<dataType>>

#### 6.3.4.1	Definition

This data type represents the properties of network slice subnet related requirement that should be supported by the network slice subnet instance in a 5G network.

#### 6.3.4.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| sliceProfileId | M | T | F | T | T |
| sNSSAIList | M | T | T | F | T |
| pLMNIdList | M | T | T | F | T |
| perfReq | M | T | T | F | T |
| maxNumberofUEs | O | T | T | F | T |
| coverageAreaTAList | O | T | T | F | T |
| latency | O | T | T | F | T |
| uEMobilityLevel | O | T | T | F | T |
| resourceSharingLevel | O | T | T | F | T |

#### 6.3.4.3	Attribute constraints

None.

#### 6.3.4.4	Notifications

The clause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.5	NsInfo <<dataType>>

#### 6.3.5.1	Definition

This data type represents the properties of network service information (See clause 8.3.3.2.2 of ETSI GS NFV-IFA 013 [29]) corresponding to the network slice subnet instance.

#### 6.3.5.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nSInstanceId | M | T | F | F | T |
| nsName | O | T | F | F | T |
| description | O | T | F | F | T |

#### 6.3.5.3	Attribute constraints

None.

#### 6.3.5.4	Notifications

The clause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.6	ServAttrCom <<dataType>>

#### 6.3.6.1	Definition

This data type represents the common properties of service requirement related attributes (see GSMA NG.116 [50] corresponding to Attribute categories, tagging and exposure).

#### 6.3.6.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| category | M | T | F | F | T |
| tagging | CM | T | F | F | T |
| exposure | M | T | F | F | T |

#### 6.3.6.3	Attribute constraints

| Name | Definition |
| --- | --- |
| tagging Support Qualifier | Condition: It shall be supported if the category is character. Otherwise this attribute shall be absent. |

#### 6.3.6.4	Notifications

The clause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.7	DelayTolerance<<dataType>>

#### 6.3.7.1	Definition

This data type represents the delay tolerance (See Clause 3.4.3 of GSMA NG.116 [50]).

#### 6.3.7.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | M | T | F | F | T |
| support | M | T | F | F | T |

#### 6.3.7.3	Attribute constraints

None.

#### 6.3.7.4	Notifications

The clause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.7	DeterminComm <<dataType>>

#### 6.3.7.1	Definition

This data type represents the properties of the deterministic communication for periodic user traffic. Periodic traffic refers to the type of traffic with periodic transmissions.

#### 6.3.7.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | M | T | F | F | T |
| availability | M | T | F | F | T |
| periodicityList | M | T | T | F | T |

#### 6.3.7.3	Attribute constraints

None.

#### 6.3.7.4	Notifications

The clause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.8	DLThpt<<dataType>>

#### 6.3.8.1	Definition

This data type represents the downlink throughput per slice or per UE (See Clause 3.4.5 and 3.4.6 of GSMA NG.116 [50]).

#### 6.3.8.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | M | T | F | F | T |
| guaThpt | M | T | F | F | T |
| maxThpt | C | T | F | F | T |

#### 6.3.8.3	Attribute constraints

None.

#### 6.3.8.4	Notifications

The clause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.9	ULThpt<<dataType>>

#### 6.3.9.1	Definition

This data type represents the uplink throughput per slice or per UE (See Clause 3.4.31 and 3.4.32 of GSMA NG.116 [50]).

#### 6.3.9.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | M | T | F | F | T |
| guaThpt | O | T | F | F | T |
| maxThpt | O | T | F | F | T |

#### 6.3.9.3	Attribute constraints

None.

#### 6.3.9.4	Notifications

The clause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.10	MaxPktSize <<dataType>>

#### 6.3.10.1	Definition

This data type represents the maximum packet size (See Clause 3.4.11 of GSMA NG.116 [50]).

#### 6.3.10.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | M | T | F | F | T |
| maxSize | M | T | F | F | T |

#### 6.3.10.3	Attribute constraints

None.

#### 6.3.10.4	Notifications

The clause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.11	MaxNumberofConns <<dataType>>

#### 6.3.11.1	Definition

This data type represents maximun number of connections (See Clause 3.4.15 of GSMA NG.116 [50]).

#### 6.3.11.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | M | T | F | F | T |
| nOofConn | M | T | F | F | T |

#### 6.3.11.3	Attribute constraints

None.

#### 6.3.11.4	Notifications

The clause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.12	Void

### 6.3.13	KPIMonitoring <<dataType>>

#### 6.3.13.1	Definition

This data type represents performance monitoring (See Clause 3.4.17 of GSMA NG.116 [50]).

#### 6.3.13.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | M | T | F | F | T |
| kPIList | M | T | F | F | T |

#### 6.3.13.3	Attribute constraints

None.

#### 6.3.13.4	Notifications

The clause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.14	UserMgmtOpen<<dataType>>

#### 6.3.14.1	Definition

This data type represents User management openness (See Clause 3.4.33 of GSMA NG.116 [50]).

#### 6.3.14.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | M | T | F | F | T |
| support | M | T | F | F | T |

#### 6.3.14.3	Attribute constraints

None.

#### 6.3.14.4	Notifications

The clause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.15	V2XCommMode<<dataType>>

#### 6.3.15.1	Definition

This data type represents V2X communication mode (See Clause 3.4.35 of GSMA NG.116 [50]).

#### 6.3.15.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | M | T | F | F | T |
| v2XMode | M | T | F | F | T |

#### 6.3.15.3	Attribute constraints

None.

#### 6.3.15.4	Notifications

The clause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.16	TermDensity<<dataType>>

#### 6.3.16.1	Definition

This data type represents Terminal density (See Clause 3.4.30 of GSMA NG.116 [50]).

#### 6.3.16.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | M | T | F | F | T |
| density | M | T | F | F | T |

#### 6.3.16.3	Attribute constraints

None.

#### 6.3.16.4	Notifications

The clause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.17	EP_Transport

#### 6.3.17.1	Definition

This IOC represents the logical transport interface or endpoint which including transport level information, e.g. transport address, reachability information and QoS profiles, etc.

The IOC is inherited from Top IOC.

#### 6.3.17.2	Attributes

The EP_Transport IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| ipAddress | M | T | F | F | T |
| logicInterfaceId | M | T | T | F | T |
| nextHopInfo | O | T | F | F | T |
| qosProfile | O | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| epApplicationRef | M | T | T | F | T |

#### 6.3.17.3	Attribute constraints

None.

#### 6.3.17.4	Notifications

The common notifications defined in clause 6.5 are valid for this IOC, without exceptions or additions.

### 6.3.18	EP_Application <<ProxyClass>>

#### 6.3.18.1	Definition

This represents  <<IOC>>EP_N3 or <<IOC>>EP_NgU.

#### 6.3.18.2	Attributes

See that defined in <<IOC>>EP_N3 or <<IOC>>EP_NgU.

#### 6.3.18.3	Attribute constraints

See respective IOCs.

#### 6.3.18.4	Notifications

See respective IOCs.

## 6.4	Attribute definition

### 6.4.1	Attribute properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| availability | This parameter specifies the communication service availability requirement, expressed as a percentage. The communication service availability is defined in clause 3.1 of TS 22.261 [28]. | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: True |
| serviceProfileId | A unique identifier of property of network slice related requirement should be supported by the network slice instance. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| sliceProfileId | A unique identifier of the property of network slice subnet related requirement should be supported by the network slice subnet instance. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| operationalState | It indicates the operational state of the network slice instance or the network slice subnet instance. It describes whether or not the resource is physically installed and working. allowedValues: "ENABLED", "DISABLED". The meaning of these values is as defined in TS 28.625 [17] and ITU-T X.731 [18]. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| administrativeState | It indicates the administrative state of the network slice instance or the network slice subnet instance. It describes the permission to use or prohibition against using the instance, imposed through the OAM services. allowedValues: "LOCKED", "UNLOCKED", SHUTTINGDOWN" The meaning of these values is as defined in TS 28.625 [17] and ITU-T X.731 [18]. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| nsInfo | This attribute contains the NsInfo of the NS instance corresponding to the network slice subnet instance. The NsInfo is described in clause 8.3.3.2.2 of ETSI GS NFV-IFA 013 [29]. | type: NsInfo multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: No default value isNullable: True |
| nSInstanceId | This attribute specifies the identifier of NS instance corresponding to the network slice subnet instance. See clause 8.3.3.2.2 of ETSI GS NFV-IFA 013 [29]. | type: String multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: No default value isNullable: True |
| nsName | This attribute specifies the name of NS instance corresponding to the network slice subnet instance. See clause 8.3.3.2.2 of ETSI GS NFV-IFA 013 [29]. | type: String multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: No default value isNullable: True |
| description | This attribute specifies the description of NS instance corresponding to the network slice subnet instance. See clause 8.3.3.2.2 of ETSI GS NFV-IFA 013 [29]. | type: String multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: No default value isNullable: True |
| category | This attribute specifies the category of a service requirement/attribute of GST (see GSMA NG.116 [50]). allowedValues: character, scalability | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| tagging | This attribute specifies the tagging of a service requirement/attribute of GST in character catogary (see GSMA NG.116 [50]). allowedValues: performance, function, operation | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| exposure | This attribute specifies exposure mode of a service requirement/attribute of GST (see GSMA NG.116 [50]). allowedValues: API, KPI | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| sNSSAIList | This parameter specifies the S-NSSAI list to be supported by the new NSI to be created or the existing NSI to be re-used. sNSSAList is defined in clause 4.4.1 |  |
| perfReq | This parameter specifies the requirements to the network slice subnet in terms of the scenarios defined in the TS 22.261 [28] and TS 22.104 [51], i.e. the "performance requirements for high data rate and traffic density scenarios" in TS 22.261 [28], "periodic deterministic communication, aperiodic deterministic communication, non-deterministic communication, and mixed traffic" in TS 22.104 [51]. It is a structure containing the following elements: - list of perfReq Depending on the sST value, the list of perfReq will be - list of eMBBPerfReq or - list of uRLLCPerfReq or - list of mIoTPerfReq NOTE 1: the list of mIoTPerfReq is not addressed in the present document. allowedValues: - list of eMBBPerfReq is a list of entries where an entry identifies the performance requirements to the network slice subnet in terms of the scenarios defined in the Table 7.1-1 of TS 22.261 [28]. An entry has the following attributes: expDataRateDL (Integer), expDataRateUL (Integer), areaTrafficCapDL (Integer), areaTrafficCapUL (Integer), overallUserDensity (Integer), activityFactor (Integer), (see table 7.1-1 of TS 22.261 [28]). - list of uRLLCPerfReq is a list of entries where an entry identifies the performance requirements to the network slice subnet in terms of the scenarios defined in clauses 5.2 through 5.5 of TS 22.104 [51]. An entry has the following attributes: cSAvailabilityTarget (Float), cSReliabilityMeanTime (String), , expDataRate (Integer), msgSizeByte (String), transferIntervalTarget (String), survivalTime (String), , , (see table 5.2-1, table 5.3-1, table 5.4-1 and table 5.5-1 of TS 22.104 [51]). NOTE 2: Limitation on attribute values in instances of SliceProfile is not addressed in the present document. NOTE 3: The attributes inside perfReq here need further breaking down to define requirements for each subnetwork under different SST values. | type: PerfReq multiplicity: *1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| maxNumberofUEs | An attribute specifies the maximum number of UEs may simultaneously access the network slice instance. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| coverageAreaTAList | An attribute specifies a list of TrackingAreas where the NSI can be selected. allowedValues: Legacy TAC and Extended TAC are defined in clause 9.3.3.10 of TS 38.413 [5]. | type: Integer multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| latency | An attribute specifies the packet transmission latency (millisecond) through the RAN, CN, and TN part of 5G network and is used to evaluate utilization performance of the end-to-end network slice instance. See clause 6.3.1 of 28.554 [27]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| uEMobilityLevel | An attribute specifies the mobility level of UE accessing the network slice instance. See 6.2.1 of TS 22.261 [28]. allowedValues: stationary, nomadic, restricted mobility, fully mobility. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: True |
| serviceProfile.resourceSharingLevel | An attribute specifies whether the resources to be allocated to the network slice instance may be shared with another network slice instance(s). allowedValues: shared, non-shared. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: Yes isNullable: True |
| sliceProfile.resourceSharingLevel | An attribute specifies whether the resources to be allocated to the network slice subnet instance may be shared with another network slice subnet instance(s). allowedValues: shared, non-shared. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: Yes isNullable: True |
| serviceProfileList | An attribute specifies a list of ServiceProfile (see clause 6.3.3) supported by the network slice instance | type: ServiceProfile multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| sliceProfileList | An attribute specifies a list of SliceProfile (see clause 6.3.4) supported by the network slice subnet instance | type: SliceProfile multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| sST | This parameter specifies the slice/service type for a ServiceProfile. See clause 5.15.2 of TS 23.501 [2]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| delayTolerance | An attribute specifies the properties of service delivery flexibility, especially for the vertical services that are not chasing a high system performance. See clause 4.3 of TS 22.104 [51]. | type: DelayTolerance multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| DelayTolerance.support | An attribute specifies whether or not the NSI supports service delivery flexibility, especially for the vertical services that are not chasing a high system performance. allowedValues: "NOT SUPPORTED", "SUPPORTED". | type: <<enumeration>> multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| deterministicComm | An attribute specifies the properties of the deterministic communication for periodic user traffic, see clause 4.3 of TS 22.104 [51]. | type: <<DeterminComm>> multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| DeterminComm.availability | An attribute specifies whether or not the NSI supports deterministic communication for period user traffic. allowedValues: "NOT SUPPORTED", "SUPPORTED". | type: <<enumeration>> multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| DeterminComm.periodicityList | An attribute specifies a list of periodicities supported by the NSI for deterministic communication. | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| dLThptPerSlice | This attribute defines achievable data rate of the network slice in downlink that is available ubiquitously across the coverage area of the slice, refer NG.116 [50]. | type: DLThpt multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| dLThptPerUE | This attribute defines data rate supported by the network slice per UE, refer NG.116 [50]. | type: DLThpt multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| guaThpt | This attribute describes the guaranteed data rate. | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| maxThpt | This attribute describes the maximum data rate. | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| uLThptPerSlice | This attribute defines achievable data rate of the network slice in uplink that is available ubiquitously across the coverage area of the slice, refer NG.116 [50]. | type: ULThpt multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| uLThptPerUE | This attribute defines data rate supported by the network slice per UE, refer NG.116 [50]. | type: ULThpt multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| maxPktSize | This parameter specifies the maximum packet size supported by the network slice, refer NG.116 [50]. | type: MaxPktSize multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| MaxPktSize.maxsize | This parameter specifies the maximum packet size supported by the network slice, refer NG.116 [50]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| maxNumberofConns | This parameter defines the maximum number of concurrent sessions supported by the network slice, refer NG.116 [50]. | type: MaxNumberofConns multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| MaxNumberofConns.nOofConn | This parameter defines the maximum number of concurrent sessions supported by the network slice, refer NG.116 [50]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| kPIMonitoring | An attribute specifies the name list of KQIs and KPIs available for performance monitoring. | type: KPIMonitoring multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| KPIMonitoring. kPIList | An attribute specifies the name list of KQIs and KPIs available for performance monitoring. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| userMgmtOpen | An attribute specifies whether or not the NSI supports the capability for the NSC to manage their users or groups of users' network services and corresponding requirements. | type: UserMgmtOpen multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| UserMgmtOpen.support | An attribute specifies whether or not the NSI supports the capability for the NSC to manage their users or groups of users' network services and corresponding requirements. allowedValues: "NOT SUPPORTED", "SUPPORTED". | type: <<enumeration>> multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| v2XCommModels | An attribute specifies whether or not the V2X communication mode is supported by the NSI. | type: V2XCommMode multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| V2XCommMode.v2XMode | An attribute specifies whether or not the V2X communication mode is supported by the NSI. allowedValues: "NOT SUPPORTED", "SUPPORTED BY NR". | type: <<enumeration>> multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| coverageArea | An attribute specifies the coverage area of the network slice, i.e. the geographic region where a 3GPP communication service is accessible, see Table 7.1-1 of TS 22.261 [28]) and NG.116 [50]. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| termDensity | An attribute specifies the overall user density over the coverage area of the network slice. See Table 7.1-1 of TS 22.261 [28]). | type: TermDensity multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| TermDensity.density | An attribute specifies the overall user density over the coverage area of the network slice. See Table 7.1-1 of TS 22.261 [28]). | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| activityFactor | An attribute specfies the percentage value of the amount of simultaneous active UEs to the total number of UEs where active means the UEs are exchanging data with the network. See Table 7.1-1 of TS 22.261 [28]). | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| uESpeed | An attribute specifies the maximum speed (in km/hour) supported by the network slice at which a defined QoS can be achieved. See Table 7.1-1 of TS 22.261 [28]). | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| jitter | An attribute specifies the deviation from the desired value to the actual value when assessing time parameters, see clause C.4.1 of TS 22.104 [51]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| survivalTime | An attribute specifies the time that an application consuming a communication service may continue without an anticipated message. See clause 5 of TS 22.104 [51]). | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| reliability | An attribute specifies in the context of network layer packet transmissions, percentage value of the amount of sent network layer packets successfully delivered to a given system entity within the time constraint required by the targeted service, divided by the total number of sent network layer packets, see TS 22.261 [28] and TS 22.104 [51]. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| NetworkSlice.networkSliceSubnetRef | This holds a DN of NetworkSliceSubnet relating to the NetworkSlice instance. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NetworkSliceSubnet.networkSliceSubnetRef | This holds a list of DN of constituent NetworkSliceSubnet supporting NetworkSliceSubnet instance | type: DN multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| managedFunctionRef | This holds a list of DN of ManagedFunction instances supporting the NetworkSliceSubnet instance. | type: DN multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| ipAddress | This parameter specifies the IP address assigned to a logical transport interface/endpoint. It can be an IPv4 address (See RFC 791 [37]) or an IPv6 address (See RFC 2373 [38]). See note 1 | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| logicInterfaceId | This parameter specifies the identify of a logical transport interface. It could be VLAN ID (See IEEE 802.1Q [39]), MPLS Tag or Segment ID. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nextHopInfoList | This parameter is used to identify ingress transport node. Each node can be identified by any of combination of IP address of next-hop router of transport network, system name, port name, IP management address of transport nodes. | type: String multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| qosProfileRefList | This parameter specifies reference to QoS Profile for a logical transport interface. A QoS profile includes a set of parameters which are locally provisioned on both sides of a logical transport interface. | type: String multiplicity: * isOrdered: N/A isUnique: True defaultValue: None isNullable: True |
| epApplicationRef | This parameter specifies a list of application level EPs associated with the logical transport interface. See note 2. | type: DN multiplicity: 1..* isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| epTransportRef | This parameter specifies a list of transport level EPs associated with the application level EP | type: DN multiplicity: * isOrdered: N/A isUnique: True defaultValue: None isNullable: True |
| NOTE 1: There is no direct relationship between localAddress/remoteAddress in EP_RP and ipAddress in EP_transport. While the localAddress/remoteAddress in EP_RP could be exchanged as part of signalling between GTP-u tunnel end points, ipAddress in EP_transport is used for transport routing. NOTE 2: Application level EP represents EP_RP defined in TS 28.622 (see [30]). e.g. including EP_NgC, EP_N3, etc... |  |  |

## 6.5	Common notifications

### 6.5.1	Alarm notifications

This clause presents a list of notifications, defined in TS 28.532 [35], that an MnS consumer may receive. The notification header attribute objectClass/objectInstance shall capture the DN of an instance of a class defined in the present document.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyNewAlarm | M | -- |
| notifyClearedAlarm | M | -- |
| notifyAckStateChanged | M | -- |
| notifyAlarmListRebuilt | M | -- |
| notifyChangedAlarm | O | -- |
| notifyCorrelatedNotificationChanged | O | -- |
| notifyChangedAlarmGeneral | O | -- |
| notifyComments | O | -- |
| notifyPotentialFaultyAlarmList | O | -- |

### 6.5.2	Configuration notifications

This clause presents a list of notifications, defined in TS 28.532 [35], that an MnS consumer may receive. The notification header attribute objectClass/objectInstance shall capture the DN of an instance of a class defined in the present document.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyMOICreation | O | -- |
| notifyMOIDeletion | O | -- |
| notifyMOIAttributeValueChanges | O | -- |
| notifyEvent | O | -- |

# 7	Solution Set (SS)

The present document defines the following NRM Solution Set definitions for NR and NG-RAN:

-	XML based 3GPP NR and NG-RAN NRM Solution Set (Annex C).

-	JSON based 3GPP NR and NG-RAN NRM Solution Set (Annex D).

-	YANG based 3GPP NR and NG-RAN NRM Solution Set (Annex E).

The present document defines the following NRM Solution Set definitions for 5GC:

-	XML based 3GPP 5GC NRM Solution Set (Annex F).

-	JSON based 3GPP 5GC NRM Solution Set (Annex G).

-	YANG based 3GPP 5GC NRM Solution Set (Annex H).

The present document defines the following NRM Solution Set definitions for network slice and network slice subnet:

-	XML based 3GPP Network Slice NRM Solution Set (Annex I).

-	JSON based 3GPP Network Slice NRM Solution Set (Annex J).

Annex A (normative):
Cell state handling

# A.1	Relation between the administrative state and the "Pre-operation state of the gNB-DU Cell"

The administrative state indicates the permission to use or prohibition against using the cell, imposed through the OAM services. The administrative state has three values: "LOCKED", "SHUTTING DOWN" or "UNLOCKED"

The meanings of these values are defined in ITU-T Recommendation X.731 [18].

The relation between the administrative state and the "Pre-operation state of the gNB-DU Cell" is defined in clause 8.5 of TS 38.401 [4]. See below an extract from clause 8.5 of TS 38.401 [4] on the F1 startup and cell activation.

If the operationalState is "ENABLED" (i.e. the resource is physically installed and working) and if the administrativeState is "UNLOCKED", the step "0: Pre-operational state" will exit and the step "1: F1 Setup Request" will be executed."

8.5	F1 Startup and cells activation

This function allows to setup the F1 interface between a gNB-DU and a gNB-CU and it allows to activate the gNB-DU cells.

Figure 8.5-1: F1 startup and cell activation

# A.2	Combined state diagram for gNB cell

This is the Combined state diagram for gNB cell.

Figure A.2-1: Combined gNB cell state diagram

The gNB-DU maintains cell states. The following table is the gNB cell state transition table.

In 3-split and 2-split deployment scenarios, the interactions between gNB-CU and gNB-DU are standardized. The interactions specified under the column "The state transition events and actions" of "The gNB Cell state transition table" below shall be present for the state transition.

In the non-split deployment scenarios, the interactions between gNB-CU and gNB-DU are not standardized. The interactions between gNB-CU and gNB-DU specified under the column "The state transition events and actions" of "The gNB Cell state transition table" can be replaced by other means that is not standardized.

Table A.2-1: The gNB Cell state transition table

| Transition number | The state transition event and actions |
| --- | --- |
| 1 | Event: Receive request to unlock. Action: None. |
| 2 | Event: Receive request to lock. Action: None. |
| 2a | Event: Receive request to lock Action: Send to gNB-CU the "gNB-DU Configuration Update message" with served cell to delete. |
| 3 | Event: When the required cell resource is physically installed and working. Action: none. |
| 4 | Event: When the required cell resource is not physically installed or is not working. Action: Send to gNB-CU the "gNB-DU Configuration update message" with cell to delete. |
| 4a | Event: When the required cell resource is physically uninstalled or is not working. Action: Send to gNB-CU the "GNB-DU Configuration Update message" with served cell to delete. |
| 5 | Event: Receive from gNB-CU the "F1 Setup Response message" (identifying the cell to be activated). The cell is activated successfully. Actions: Do nothing or send gNB-CU the "gNB-DU Configuration Update message" with Cell stated as active' ----- or ----- Event: Receive from gNB-CU the "gNB-CU Configuration Update message" (identifying cell to be activated e.g., in case that the cell was not activated using the "F1 Setup Response message"). Actions: The cell is activated successfully. Send to gNB-CU the "gNB-CU Configuration Update Response" to confirm the cell is in active state. ----- or ----- Event: Receive from gNB-CU the "gNB-DU Configuration Update Acknowledge message" (identifying cell to be activated e.g., in case that the cell was not activated using the "F1 Setup Response message") and the cell is activated successfully Actions: Do nothing. |
| 6 | Event: Receive from gNB-CU the "gNB-CU Configuration Update message" and responds with gNB-CU Configuration Update Acknowledge messages. Actions: Respond with gNB-CU Configuration Update Acknowledge messages. ----- or ----- Event: Event: DU experiences an internal failure and decided to place the cell into inactive state. Actions: Send to gNB-CU the "gNB-DU Cell status Update message" |
| 7 | Event: Send to gNB-CU the "F1 Setup request" (identifying the cell that is configured and ready to be activated). Actions: none. ----- or ----- Send to gNB-CU the "gNB-DU Configuration Update message" with the served cell to add. Actions: none. |
| 8 | Event: Sends to gNB-CU the "gNB-DU Configuration Update message" with served cell to delete. Receive response from gNB-CU the "gNB-DU Configuration Update Acknowledge message". Actions: None. |
| 9 | Event: Receive request to shut down. Actions: None. |
| 10 | Event: Last user quit. Actions: Send to gNB-CU the "GNB-DU Configuration Update message" with served cell to delete. |
| 11 | Event: When a cell is created and is configured. Actions: None |
| 12 | Event: When a cell is deleted. Action: None. |

Annex B (normative): 
NSI and NSSI state handling

# B.1	NSI state handling

An NSI is a logical object in the management system that represents a complex grouping of resources that may be in various states. At any time, the management system needs to know the state of an NSI.

The ITU-T X.731 [18], to which [17] refers, has defined the inter-relation between the administrative state, operational state and usage state of systems in general.

Figure B.1: Combined NSI state diagram

In an NSI deployment scenario, the interactions between communication service management function, network slice management function and network slice subnet management function are standardized. The interactions specified under the column "The state transition events and actions" of "NSI state transition table" below shall be present for the state transition.

Figure B.2: NSI state diagram with state transition triggers

Table B.1: The NSI state transition table

| Trigger number | The state transition events and actions |
| --- | --- |
| 0 | NSMF responds positively to the "Create NSI request" message, the NSI is created and the state is set to Locked |
| 1 | NSMF responds positively to the "Activate NSI request" message (identifying the NSI to be activated). ----- or ------ CM operation to set administrative state to Unlocked. |
| 1a | CM Operation to set administrative state to Unlocked |
| 2 | The last user of the NSI stops using the NSI |
| 2a | CM Operation to set administrative state to Shutting down |
| 3 | When the NSI and its constituents are installed and working NSMF receives positive response to the "Allocate NSSI" message (applicable to the NSI to be enabled). |
| 4 | When the NSI or its constituents are not installed or not working NSMF receives positive response to the "Deallocate NSSI" message (applicable to the NSI to be disabled) |
| 5 | NSMF responds positively to the "Deallocate NSI request" message, the NSI is deleted and the state is set to NULL |

# B.2	State handling of NSSI

An NSSI is a logical object in the management system that represents a complex grouping of resources that may be in various states. At any time the management system needs to know the state of an NSSI.

The ITU-T X.731 [18], to which [17] refers, has defined the inter-relation between the administrative state, operational state and usage state of systems in general.

Figure B.2.1: Combined NSSI state diagram

In an NSSI deployment scenario, the interactions between CSMF, NSMF and NSSMF are standardized. The interactions specified under the column "The state transition events and actions" of "NSSI state transition table" below shall be present for the state transition.

Figure B.2.2: NSSI state diagram with state transition triggers

Table B.2.1: The NSSI state transition table

| Trigger number | The state transition events and actions |
| --- | --- |
| 0 | NSSMF responds positively to the "Create NSSI request" message, the NSSI is created and the state is set to Locked |
| 1 | NSSMF responds positively to the "Activate NSSI request" message (identifying the NSSI to be activated). ----- or ------ CM operation to set administrative state to Unlocked. |
| 1a | CM Operation to set administrative state to Unlocked |
| 2 | The last user of the NSSI stops using the NSSI |
| 2a | CM Operation to set administrative state to Shutting down |
| 3 | When the NSSI constituents are installed and working NSSMF receives positive response to the "Create NSSI constituent" message (applicable to the NSSI to be enabled). |
| 4 | When the NSSI constituents are not installed or not working NSSMF receive positive response to the "Delete NSSI constituent" message (applicable to the NSSI to be disabled) |
| 5 | NSSMF responds positively to the "Delete NSSI request" message, the NSSI is deleted and the state is set to NULL. |

Annex C (normative):
XML definitions for NR NRM

# C.1	General

This annex contains the XML definitions for the NR and NG-RAN NRM, in accordance with NR and NG-RAN NRM Information Model definitions specified in clause 4.

# C.2	Architectural features

The overall architectural feature of NR NRM information model is specified in clause 4, this clause specifies features that are specific to the Schema definitions.

The XML definitions of the present document specify the schema for a configuration content, which can be included in a configuration file for Bulk configuration management operations

# C.3	Mapping

## C.3.1	General mapping

An IOC maps to an XML element of the same name as the IOC's name in the Information Model. An IOC attribute maps to a sub-element of the corresponding IOC's XML element, and the name of this sub-element is the same as the attribute's name in the Information Model.

## C.3.2	Information Object Class (IOC) mapping

The mapping is not present in the current version of the present document.

# C.4	Solution Set definitions

## C.4.1	XML definition structure

The overall description of the file format of configuration data XML files is provided by TS 32.616 [33].

The present document defines the NRM-specific XML schema nrNrm.xsd for the NR NRM Information Model defined in clause 4.

XML schema nrNrm.xsd explicitly declares NRM-specific XML element types for the related NRM.

The definition of those NRM-specific XML element types complies with the generic mapping rules defined in TS 32.616 [33].

## C.4.2	Graphical representation

The graphical representation is not present in the current version of the present document.

## C.4.3	XML schema "nRNrm.xsd"

<?xml version="1.0" encoding="UTF-8"?>

<!--

TS 28.541 NR Network Resource Model

XML schema definition

nrNrm.xsd

-->

<schema xmlns="http://www.w3.org/2001/XMLSchema"

xmlns:xn="http://www.3gpp.org/ftp/specs/archive/28_series/28.623#genericNrm"

xmlns:nn="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#nrNrm"

xmlns:en="http://www.3gpp.org/ftp/specs/archive/28_series/28.659#eutranNrm"

xmlns:epc="http://www.3gpp.org/ftp/specs/archive/28_series/28.709#epcNrm"

xmlns:sm="http://www.3gpp.org/ftp/specs/archive/28_series/28.626#stateManagementIRP"

xmlns:ngc="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#ngcNrm"

xmlns:sp="http://www.3gpp.org/ftp/specs/archive/28_series/28.629#sonPolicyNrm"

targetNamespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#nrNrm" elementFormDefault="qualified">

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.623#genericNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.709#epcNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.626#stateManagementIRP"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#ngcNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.629#sonPolicyNrm"/>

<simpleType name="GnbId">

<restriction base="unsignedLong">

<maxInclusive value="4294967295"/>

</restriction>

</simpleType>

<simpleType name="GnbIdLength">

<restriction base="integer">

<minLength value="22"/>

<maxLength value="32"/>

</restriction>

</simpleType>

<simpleType name="Nci">

<restriction base="unsignedLong">

<maxInclusive value="68719476735"/>

</restriction>

</simpleType>

<simpleType name="Pci">

<restriction base="unsignedShort">

<maxInclusive value="503"/>

<!-- Minimum value is 0, maximum value is 3x167+2=503 -->

</restriction>

</simpleType>

<simpleType name="NrTac">

<restriction base="unsignedLong">

<maxInclusive value="16777215"/>

<!--5G TAC is 3-octets length -->

</restriction>

</simpleType>

<simpleType name="GnbDuId">

<restriction base="unsignedLong">

<maxInclusive value="68719476735"/>

<!-- Minimum value is 0, maximum value is 2^36-1=68719476735 -->

</restriction>

</simpleType>

<simpleType name="GnbCuupId">

<restriction base="unsignedLong">

<maxInclusive value="68719476735"/>

<!-- Minimum value is 0, maximum value is 2^36-1=68719476735 -->

</restriction>

</simpleType>

<simpleType name="GnbName">

<restriction base="string">

<minLength value="1"/>

<maxLength value="150"/>

</restriction>

</simpleType>

<simpleType name="CyclicPrefix">

<restriction base="integer">

<enumeration value="15"/>

<enumeration value="30"/>

<enumeration value="60"/>

<enumeration value="120"/>

</restriction>

</simpleType>

<simpleType name="QuotaType">

<restriction base="string">

<enumeration value="STRICT"/>

<enumeration value="FLOAT"/>

</restriction>

</simpleType>

<simpleType name="CellState">

<restriction base="string">

<enumeration value="IDLE"/>

<enumeration value="INACTIVE"/>

<enumeration value="ACTIVE"/>

</restriction>

</simpleType>

<simpleType name="BwpContext">

<restriction base="string">

<enumeration value="DL"/>

<enumeration value="UL"/>

<enumeration value="SUL"/>

</restriction>

</simpleType>

<simpleType name="IsInitialBwp">

<restriction base="string">

<enumeration value="INITIAL"/>

<enumeration value="OTHER"/>

</restriction>

</simpleType>

<simpleType name="qOffsetRangeList">

<restriction base="string">

<enumeration value="dB-24"/>

<enumeration value="dB-22"/>

<enumeration value="dB-20"/>

<enumeration value="dB-18"/>

<enumeration value="dB-16"/>

<enumeration value="dB-14"/>

<enumeration value="dB-12"/>

<enumeration value="dB-10"/>

<enumeration value="dB-8"/>

<enumeration value="dB-6"/>

<enumeration value="dB-5"/>

<enumeration value="dB-4"/>

<enumeration value="dB-3"/>

<enumeration value="dB-2"/>

<enumeration value="dB-1"/>

<enumeration value="dB0"/>

<enumeration value="dB1"/>

<enumeration value="dB2"/>

<enumeration value="dB3"/>

<enumeration value="dB4"/>

<enumeration value="dB5"/>

<enumeration value="dB6"/>

<enumeration value="dB8"/>

<enumeration value="dB10"/>

<enumeration value="dB12"/>

<enumeration value="dB14"/>

<enumeration value="dB16"/>

<enumeration value="dB18"/>

<enumeration value="dB20"/>

<enumeration value="dB22"/>

<enumeration value="dB24"/>

</restriction>

</simpleType>

<simpleType name="isESCoveredBy">

<restriction base="string">

<enumeration value="NO"/>

<enumeration value="PARTIAL"/>

<enumeration value="FULL"/>

</restriction>

</simpleType>

<simpleType name="cellReselectionPriority">

<restriction base="unsignedLong">

<minInclusive value="0"/>

<maxInclusive value="16"/>

<!--Value 0 means lowest priority-->

</restriction>

</simpleType>

<simpleType name="cellReselectionSubPriority">

<restriction base="unsignedLong">

<minInclusive value="0"/>

<maxInclusive value="16"/>

<!--Value 0 means lowest priority-->

</restriction>

</simpleType>

<simpleType name="PMaxRangeType">

<restriction base="short">

<minInclusive value="-30"/>

<maxInclusive value="33"/>

</restriction>

</simpleType>

<simpleType name="qOffsetFreq">

<restriction base="short">

<minInclusive value="-24"/>

<maxInclusive value="24"/>

</restriction>

</simpleType>

<simpleType name="qQualMin">

<restriction base="integer">

<minInclusive value="-34"/>

<maxInclusive value="0"/>

</restriction>

</simpleType>

<simpleType name="qRxLevMin">

<restriction base="integer">

<minInclusive value="-140"/>

<maxInclusive value="-44"/>

</restriction>

</simpleType>

<simpleType name="Thresxhighp">

<restriction base="integer">

<minInclusive value="0"/>

<maxInclusive value="62"/>

</restriction>

</simpleType>

<simpleType name="Threshxhighq">

<restriction base="integer">

<minInclusive value="0"/>

<maxInclusive value="31"/>

</restriction>

</simpleType>

<simpleType name="Threshxlowp">

<restriction base="integer">

<minInclusive value="0"/>

<maxInclusive value="62"/>

</restriction>

</simpleType>

<simpleType name="Threshxlowq">

<restriction base="integer">

<minInclusive value="0"/>

<maxInclusive value="62"/>

</restriction>

</simpleType>

<simpleType name="Treselectionnr">

<restriction base="integer">

<minInclusive value="0"/>

<maxInclusive value="7"/>

</restriction>

</simpleType>

<simpleType name="Treselectionnrsfhigh">

<restriction base="string">

<enumeration value="25"/>

<enumeration value="50"/>

<enumeration value="75"/>

<enumeration value="100"/>

</restriction>

</simpleType>

<simpleType name="Treselectionnrsfmedium">

<restriction base="string">

<enumeration value="25"/>

<enumeration value="50"/>

<enumeration value="75"/>

<enumeration value="100"/>

</restriction>

</simpleType>

<simpleType name="Absolutefrequencyssb">

<restriction base="integer">

<minInclusive value="0"/>

<maxInclusive value="3279165"/>

</restriction>

</simpleType>

<simpleType name="Ssbsubcarrierspacing">

<restriction base="string">

<enumeration value="15"/>

<enumeration value="30"/>

<enumeration value="120"/>

<enumeration value="240"/>

</restriction>

</simpleType>

<simpleType name="Multifrequencybandlistnr">

<restriction base="integer">

<minInclusive value="1"/>

<maxInclusive value="256"/>

</restriction>

</simpleType>

<simpleType name="beamType">

<restriction base="string">

<enumeration value="SSB-BEAM"/>

</restriction>

</simpleType>

<simpleType name="beamAzimuth">

<restriction base="integer">

<minInclusive value="-1800"/>

<maxInclusive value="1800"/>

</restriction>

</simpleType>

<simpleType name="beamTilt">

<restriction base="integer">

<minInclusive value="-900"/>

<maxInclusive value="900"/>

</restriction>

</simpleType>

<simpleType name="beamHorizWidth">

<restriction base="integer">

<minInclusive value="0"/>

<maxInclusive value="3599"/>

</restriction>

</simpleType>

<simpleType name="beamVertWidth">

<restriction base="integer">

<minInclusive value="0"/>

<maxInclusive value="1800"/>

</restriction>

</simpleType>

<simpleType name="coverageShapeType">

<restriction base="integer">

<minInclusive value="0"/>

<maxInclusive value="65535"/>

</restriction>

</simpleType>

<simpleType name="resourceType">

<restriction base="string">

<enumeration value="PRB"/>

<enumeration value="RRC"/>

<enumeration value="DRB"/>

</restriction>

</simpleType>

<complexType name="LocalEndPoint">

<sequence>

<element name="ipv4Address" type="string"/>

<element name="ipv6Address" type="string"/>

<element name="ipv6Prefix" type="string"/>

<element name="vlanId" type="integer"/>

</sequence>

</complexType>

<complexType name="RemoteEndPoint">

<sequence>

<element name="ipv4Address" type="string"/>

<element name="ipv6Address" type="string"/>

<element name="ipv6Prefix" type="string"/>

</sequence>

</complexType>

<complexType name="blackListEntry">

<sequence minOccurs="0" maxOccurs="1007">

<element name="pci" type="en:Pci" maxOccurs="504"/>

</sequence>

</complexType>

<complexType name="blackListEntryIdleMode">

<sequence minOccurs="0" maxOccurs="1007">

<element name="pci" type="en:Pci" maxOccurs="504"/>

</sequence>

</complexType>

<complexType name="PLMNIdList">

<sequence>

<element name="pLMNId" type="en:PLMNId" maxOccurs="6"/>

<!-- The first pLMNId of the pLMNIdList is primary PLMN id -->

</sequence>

</complexType>

<complexType name="cellIndividualOffset">

<sequence>

<element name="rsrpOffsetSSB" type="qOffsetRangeList"/>

<element name="rsrqOffsetSSB" type="qOffsetRangeList"/>

<element name="sinrOffsetSSB" type="qOffsetRangeList"/>

<element name="rsrpOffsetCSI-RS" type="qOffsetRangeList"/>

<element name="rsrqOffsetCSI-RS" type="qOffsetRangeList"/>

<element name="sinrOffsetCSI-RS" type="qOffsetRangeList"/>

</sequence>

</complexType>

<complexType name="PLMNInfoType">

<sequence>

<element name="pLMNId" type="en:PLMNId"/>

<element name="sNSSAI" type="ngc:SNssai" minOccurs="0"/>

</sequence>

</complexType>

<complexType name="PLMNInfoListType">

<sequence>

<element name="pLMNInfo" type="PLMNInfoType" minOccurs="1"/>

</sequence>

</complexType>

<simpleType name="maximumDeviationHoTrigger">

<restriction base="integer">

<minInclusive value="-20"/>

<maxInclusive value="20"/>

</restriction>

</simpleType>

<simpleType name="minimumTimeBetweenHoTriggerChange">

<restriction base="integer">

<minInclusive value="0"/>

<maxInclusive value="604800"/>

</restriction>

</simpleType>

<simpleType name="tstoreUEcntxt">

<restriction base="integer">

<minInclusive value="0"/>

<maxInclusive value="1023"/>

</restriction>

</simpleType>

<simpleType name="loadThreshold">

<restriction base="integer">

<minInclusive value="0"/>

<maxInclusive value="100"/>

</restriction>

</simpleType>

<simpleType name="timeDuration">

<restriction base="integer">

<minInclusive value="0"/>

<maxInclusive value="900"/>

</restriction>

</simpleType>

<simpleType name="energySavingControl">

<restriction base="string">

<enumeration value="toBeEnergySaving"/>

<enumeration value="toBeNotEnergySaving"/>

</restriction>

</simpleType>

<simpleType name="energySavingState">

<restriction base="string">

<enumeration value="isNotEnergySaving"/>

<enumeration value="isEnergySaving"/>

</restriction>

</simpleType>

<simpleType name="isProbingCapable">

<restriction base="string">

<enumeration value="yes"/>

<enumeration value="no"/>

</restriction>

</simpleType>

<simpleType name="AccessDelayRange">

<restriction base="unsignedShort">

<minInclusive value="10"/>

<maxInclusive value="560"/>

</restriction>

</simpleType>

<simpleType name="NumberOfPreambleRange">

<restriction base="unsignedShort">

<minInclusive value="1"/>

<maxInclusive value="200"/>

</restriction>

</simpleType>

<simpleType name="RachProbability">

<restriction base="unsignedShort">

<enumeration value="25"/>

<enumeration value="50"/>

<enumeration value="75"/>

<enumeration value="90"/>

</restriction>

</simpleType>

<complexType name="UeAccDelayProbilityDistPerSSB">

<sequence>

<element name="Probability" type="sp:RachProbability"/>

<element name="AccessDelay" type="sp:AccessDelayRange"/>

</sequence>

</complexType>

<complexType name="UeAccDelayProbilityDistPerSSBlist">

<sequence>

<element name="ueAccDelayProbilityDistPerSSB" type="sp:UeAccDelayProbilityDistPerSSB" maxOccurs="4"/>

</sequence>

</complexType>

<complexType name="UeAccProbilityDistPerSSB">

<sequence>

<element name="Probability" type="sp:RachProbability"/>

<element name="NumberOfPreamble" type="sp:NumberOfPreambleRange"/>

</sequence>

</complexType>

<complexType name="UeAccProbilityDistPerSSBlist">

<sequence>

<element name="ueAccProbilityDistPerSSB" type="sp:UeAccProbilityDistPerSSB" maxOccurs="4"/>

</sequence>

</complexType>

<simpleType name="NRPci">

<restriction base="unsignedShort">

<maxInclusive value="1007"/>

</restriction>

</simpleType>

<complexType name="NRPciList">

<sequence>

<element name="nRPci" type="en:NRPci" maxOccurs="1008"/>

</sequence>

</complexType>

<simpleType name="NRPci">

<restriction base="unsignedShort">

<maxInclusive value="1007"/>

</restriction>

</simpleType>

<complexType name="CSonPciList">

<sequence>

<element name="nRPci" type="en:NRPci" maxOccurs="1008"/>

</sequence>

</complexType>

<element name="GNBDUFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<!-- Inherited attributes from ManagedFunction -->

<element name="userLabel" type="string" minOccurs="0"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="peeParametersList" type="xn:peeParametersListType" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<!--End of inherited attributes from ManagedFunction-->

<element name="gnbId" type="nn:GnbId"/>

<element name="gnbIdLength" type="nn:GnbIdLength"/>

<element name="gnbDUId" type="nn:GnbDuId"/>

<element name="gnbDuName" type="nn:GnbName" minOccurs="0"/>

<element name="x2Blacklist" type="string" minOccurs="0"/>

<element name="x2Whitelist" type="string" minOccurs="0"/>

<element name="xnBlacklist" type="string" minOccurs="0"/>

<element name="xnWhitelist" type="string" minOccurs="0"/>

<element name="xnHOBlackList" type="string" minOccurs="0"/>

<element name="x2HOBlackList" type="string" minOccurs="0"/>

<element name="aggressorSetID" type="nn:AggressorSetID"/>

<element name="victimSetID" type="nn:VictimSetID"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="nn:NRCellDU"/>

<element ref="nn:BWP"/>

<element ref="nn:NRSectorCarrier"/>

<element ref="nn:EP_F1C"/>

<element ref="nn:EP_F1U"/>

</choice>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="GNBCUCPFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<!-- Inherited attributes from ManagedFunction -->

<element name="userLabel" type="string" minOccurs="0"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="peeParametersList" type="xn:peeParametersListType" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<!--End of inherited attributes from ManagedFunction-->

<element name="gnbId" type="nn:GnbId" />

<element name="gnbIdLength" type="nn:GnbIdLength"/>

<element name="gnbCuName" type=" nn:GnbName" minOccurs="0"/>

<element name="pLMNId" type="en:PLMNId" />

<element name="x2Blacklist" type="string" minOccurs="0"/>

<element name="x2Whitelist" type="string" minOccurs="0"/>

<element name="xnBlacklist" type="string" minOccurs="0"/>

<element name="xnWhitelist" type="string" minOccurs="0"/>

<element name="xnHOBlackList" type="string" minOccurs="0"/>

<element name="x2HOBlackList" type="string" minOccurs="0"/>

<element name="mappingSetIDBackhaulAddress" type="MappingSetIDBackhaulAddress" minOccurs="0"/>

<element name="configurable5QISetRef" type="xn:dn"/>

<element name="dynamic5QISetRef" type="xn:dn" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="nn:NRCellCU"/>

<element ref="nn:EP_F1C"/>

<element ref="nn:EP_E1"/>

<element ref="nn:EP_XnC"/>

<element ref="nn:EP_X2C"/>

<element ref="nn:EP_NgC"/>

<element ref="xn:VsDataContainer"/>

</choice>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="DESManagementFunction"/>

<element ref="DRACHOptimizationFunction"/>

<element ref="DMROFunction"/>

<element ref="DANRManagementFunction"/>

</choice>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="GNBCUUPFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<!-- Inherited attributes from ManagedFunction -->

<element name="userLabel" type="string" minOccurs="0"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="peeParametersList" type="xn:peeParametersListType" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<!--End of inherited attributes from ManagedFunction-->

<element name="gNBCUUPId" type="nn:GnbCuupId "/>

<element name="pLMNInfoList" type="PLMNInfoListType"/>

<element name="gNBId" type="nn:GnbId"/>

<element name="gnbIdLength" type="nn:GnbIdLength"/>

<element name="configurable5QISetRef" type="xn:dn"/>

<element name="dynamic5QISetRef" type="xn:dn" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="nn:EP_E1"/>

<element ref="nn:EP_F1U"/>

<element ref="nn:EP_XnU"/>

<element ref="nn:EP_NgU"/>

<element ref="nn:EP_X2U"/>

<element ref="nn:EP_S1U"/>

<element ref="xn:VsDataContainer"/>

</choice>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="NRCellCU">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<!-- Inherited attributes from ManagedFunction -->

<element name="userLabel" type="string" minOccurs="0"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="peeParametersList" type="xn:peeParametersListType" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<!--End of inherited attributes from ManagedFunction-->

<element name="nCGI" type="nn:Ncgi"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="sNSSAIList" type="ngc:SnssaiList" minOccurs="0"/>

<element name="nRFrequencyRef" type="xn:dn" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

<element ref="nRCellRelation"/>

<element ref="nRFreqRelation"/>

<element ref="eUtranCellRelation"/>

<element ref="eUtranFreqRelation"/>

</choice>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:MeasurementControl"/>

</choice>

<choice minOccurs="0" maxOccurs="1">

<element ref="sp:EnergySavingProperties"/>

<element ref="sp:ESPolicies"/>

</choice>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref= "RRMPolicyRatio"/>

</choice>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="DESManagementFunction"/>

<element ref="DRACHOptimizationFunction"/>

<element ref="DMROFunction"/>

<element ref="CESManagementFunction"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="NRCellDU">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<!-- Inherited attributes from ManagedFunction -->

<element name="userLabel" type="string" minOccurs="0"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="peeParametersList" type="xn:peeParametersListType" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<!--End of inherited attributes from ManagedFunction-->

<element name="nCGI" type="nn:Ncgi" minOccurs="0"/>

<element name="operationalState" type="sm:operationalStateType" minOccurs="0"/>

<element name="administrativeState" type="sm:administrativeStateType" minOccurs="0"/>

<element name="cellState" type="nn:CellState"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="sNSSAIList" type="ngc:SnssaiList" minOccurs="0"/>

<element name="nRpci" type="nn:Pci" />

<element name="nRTac" type="nn:NrTac" />

<element name="arfcnDL" type="integer"/>

<element name="arfcnUL" type="integer" minOccurs="0"/>

<element name="arfcnSUL" type="integer" minOccurs="0"/>

<element name="bSChannelBwDL" type="integer"/>

<element name="bSChannelBwUL" type="integer" minOccurs="0"/>

<element name="bSChannelBwSUL" type="integer" minOccurs="0"/>

<element name="nRFrequencyRef" type="xn:dn" minOccurs="0"/>

<element name="nRSectorCarrierRef" type="xn:dn" minOccurs="0"/>

<element name="bWPRef" type="xn:dn" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:MeasurementControl"/>

</choice>

<choice minOccurs="0" maxOccurs="1">

<element ref="sp:EnergySavingProperties"/>

<element ref="sp:ESPolicies"/>

</choice>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="RRMPolicyRatio"/>

</choice>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="DPCIConfigurationFunction"/>

<element ref="CPCIConfigurationFunction"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="NRSectorCarrier">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<!-- Inherited attributes from ManagedFunction -->

<element name="userLabel" type="string" minOccurs="0"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="peeParametersList" type="xn:peeParametersListType" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<!--End of inherited attributes from ManagedFunction-->

<element name="txDirection" type="nn:TxDirection"/>

<element name="configuredMaxTxPower" type="integer"/>

<element name="arfcnDL" type="integer" minOccurs="0"/>

<element name="arfcnUL" type="integer" minOccurs="0"/>

<element name="bSChannelBwDL" type="integer" minOccurs="0"/>

<element name="bSChannelBwUL" type="integer" minOccurs="0"/>

<element name="sectorEquipmentFunctionRef" type="xn:dn" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:MeasurementControl"/>

</choice>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

<choice minOccurs="0" maxOccurs="1">

<element ref="sp:EnergySavingProperties"/>

<element ref="sp:ESPolicies"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="BWP">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<!-- Inherited attributes from ManagedFunction -->

<element name="userLabel" type="string" minOccurs="0"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="peeParametersList" type="xn:peeParametersListType" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<!--End of inherited attributes from ManagedFunction-->

<element name="bwpContext" type="nn:BwpContext"/>

<element name="isInitialBwp" type="nn:IsInitialBwp"/>

<element name="subCarrierSpacing" type="integer"/>

<element name="cyclicPrefix" type="nn:CyclicPrefix"/>

<element name="startRB" type="integer"/>

<element name="numberOfRBs" type="integer"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:MeasurementControl"/>

</choice>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="CommonBeamformingFunction">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="coverageShape" type="coverageShapeType" minOccurs="0"/>

<element name="digitalTilt" type="beamTilt" minOccurs="0"/>

<element name="digitalAzimuth" type="beamAzimuth" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:MeasurementControl"/>

</choice>

<choice minOccurs="0" maxOccurs="1">

<element ref="sp:EnergySavingProperties"/>

<element ref="sp:ESPolicies"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="Beam">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="beamIndex" type="integer" minOccurs="0"/>

<element name="beamType" type="beamType" minOccurs="0"/>

<element name="beamAzimuth" type="beamAzimuth" minOccurs="0"/>

<element name="beamTilt" type="beamTilt" minOccurs="0"/>

<element name="beamHorizWidth" type="beamHorizWidth" minOccurs="0"/>

<element name="beamVertWidth" type="beamVertWidth" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:MeasurementControl"/>

</choice>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

<choice minOccurs="0" maxOccurs="1">

<element ref="sp:EnergySavingProperties"/>

<element ref="sp:ESPolicies"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_E1">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:RemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_XnC">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:RemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_XnU">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:RemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_NgC">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:LoacalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:RemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_NgU">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:RemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_F1C">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:RemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_F1U">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:RemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_S1U">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:RemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_X2C">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:RemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_X2U">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="nn:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="nn:RemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="NRCellRelation">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<!-- Inherited attributes from Top_ -->

<element name="id" type="string" />

<!--End of inherited attributes from Top_ -->

<element name="nRTCI" type="nn:Nrtci"/>

<element name="cellIndividualOffset" type="en:CellIndividualOffset"/>

<element name="nRFreqRelationRef" type="xn:dn" minOccurs="0"/>

<element name="adjacentNRCellRef" type="xn:dn" minOccurs="0"/>

<element name="isRemoveAllowed" type="boolean" minOccurs="0"/>

<element name="isHOAllowed" type="boolean" minOccurs="0"/>

<element name="isESCoveredBy" type="nn:isESCoveredBy" minOccurs="0"/>

<element name="isENDCAllowed" type="boolean" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

<choice minOccurs="0" maxOccurs="1">

<element ref="sp:EnergySavingProperties"/>

<element ref="sp:ESPolicies"/>

</choice>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="NRFreqRelation">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<!-- Inherited attributes from Top_ -->

<element name="id" type="string" />

<!--End of inherited attributes from Top_ -->

<element name="offsetMO" type="en:qOffsetRangeList"/>

<element name="blackListEntry" type="en:blackListEntry" minOccurs="0"/>

<element name="blackListEntryIdleMode" type="en:blackListEntryIdleMode" minOccurs="0"/>

<element name="cellReselectionPriority" type="en:cellReselectionPriority"/>

<element name="cellReselectionSubPriority" type="en:cellReselectionSubPriority"/>

<element name="pMax" type="en:PMaxRangeType" minOccurs="0"/>

<element name="qOffserFreq" type="nn:qOffserFreq" minOccurs="0"/>

<element name="qQualMin" type="en:qQualMin" minOccurs="0"/>

<element name="qRxLevMin" type="en:qRxLevMin" minOccurs="0"/>

<element name="threshXHighP" type="en:threshxhighp" minOccurs="0"/>

<element name="threshXHighQ" type="en:threshxhighq" minOccurs="0"/>

<element name="threshXLowP" type="en:threshxlowp" minOccurs="0"/>

<element name="threshXLowQ" type="en:threshxlowp" minOccurs="0"/>

<element name="tReselectionNr" type="nn:Treselectionnr" minOccurs="0"/>

<element name="tReselectionNRSfHigh" type="nn:Treselectionnrsfhigh" minOccurs="0"/>

<element name="tReselectionNRSfMedium" type="nn:Treselectionnrsfmedium" minOccurs="0"/>

<element name="nRFrequencyRef" type="xn:dn" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:MeasurementControl"/>

</choice>

<choice minOccurs="0" maxOccurs="1">

<element ref="sp:EnergySavingProperties"/>

<element ref="sp:ESPolicies"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="ExternalNRCellCU">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<!-- Inherited attributes from ManagedFunction -->

<element name="userLabel" type="string" minOccurs="0"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="peeParametersList" type="xn:peeParametersListType" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<!--End of inherited attributes from ManagedFunction -->

<element name="nCGI" type="nn:Ncgi"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="nRPCI" type="nn:Nrpci" minOccurs="0"/>

<element name="nRFrequencyRef" type="xn:dn" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:MeasurementControl"/>

</choice>

<choice minOccurs="0" maxOccurs="1">

<element ref="sp:EnergySavingProperties"/>

<element ref="sp:ESPolicies"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="ExternalGNBCUCPFunction" substitutionGroup="xn:SubNetworkOptionallyContainedNrmClass ">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<!-- Inherited attributes from ManagedFunction -->

<element name="userLabel" type="string" minOccurs="0"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="peeParametersList" type="xn:peeParametersListType" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<!--End of inherited attributes from ManagedFunction -->

<element name="gnbId" type="nn:GnbId" />

<element name="gnbIdLength" type="nn:GnbIdLength"/>

<element name="pLMNId" type="en:PLMNId" />

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:MeasurementControl"/>

</choice>

<choice minOccurs="0" maxOccurs="1">

<element ref="sp:EnergySavingProperties"/>

<element ref="sp:ESPolicies"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="RRMPolicy_">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="resourceType" type="ResourceType" />

<element name="rRMPolicyMemberList" type="PLMNInfoListType"/>

</all>

</complexType>

</element>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="RRMPolicyRatio">

<complexType>

<complexContent>

<extension base="RRMPolicy_">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="rRMPolicyMaxRatio" type="integer" minOccurs="1"/>

<element name="rRMPolicyMinRatio" type="integer" minOccurs="1"/>

<element name="rRMPolicyDedicatedRatio" type="integer" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

<choice minOccurs="0" maxOccurs="1">

<element ref="sp:EnergySavingProperties"/>

<element ref="sp:ESPolicies"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="NRFrequency" substitutionGroup="xn:SubNetworkOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<!-- Inherited attributes from ManagedFunction -->

<element name="userLabel" type="string" minOccurs="0"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="peeParametersList" type="xn:peeParametersListType" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<!--End of inherited attributes from ManagedFunction -->

<element name="absoluteFrequencySSB" type="nn:Absolutefrequencyssb" minOccurs="0"/>

<element name="sSBSubCarrierSpacing" type="nn:Ssbsubcarrierspacing" minOccurs="0"/>

<element name="multiFrequencyBandListNR" type="nn:MultifrequencyBandlistnr" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

<choice minOccurs="0" maxOccurs="1">

<element ref="sp:EnergySavingProperties"/>

<element ref="sp:ESPolicies"/>

</choice>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="MappingSetIDBackhaulAddress">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="setID" type="nn:SetId" />

<element name="backhaulAdress" type="BackhaulAddress" minOccurs="0"/>

</all>

</complexType>

</element>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="BackhaulAddress">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="gNBID" type="nn:GnbId" />

<element name="tAI" type="TAI" minOccurs="0"/>

</all>

</complexType>

</element>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="TAI">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="nRTac" type="nn:NrTac" />

<element name="pLMNId" type="en:PLMNId" />

</all>

</complexType>

</element>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="DANRManagementFunction">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="intrasystemANRManagementSwitch" type="boolean" minOccurs="0"/>

<element name="intrasystemANRManagementSwitch" type="beamType" minOccurs="0"/>

</all>

</complexType>

</element>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="DESManagementFunction">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="desSwitch" type="boolean" minOccurs="0"/>

<element name="intraRatEsActivationOriginalCellLoadParameters" type="IntraRatEsActivationOriginalCellLoadParameters " minOccurs="0"/>

<element name="intraRatEsActivationCandidateCellsLoadParameters" type="IntraRatEsActivationCandidateCellsLoadParameters" minOccurs="0"/>

<element name="intraRatEsDeactivationCandidateCellsLoadParameters" type="IntraRatEsDeactivationCandidateCellsLoadParameters" minOccurs="0"/>

<element name="esNotAllowedTimePeriod" type="EsNotAllowedTimePeriod" minOccurs="0"/>

<element name="interRatEsActivationOriginalCellParameters" type="InterRatEsActivationOriginalCellParameters" minOccurs="0"/>

<element name="interRatEsActivationCandidateCellParameters" type="InterRatEsActivationCandidateCellParameters" minOccurs="0"/>

<element name="interRatEsDeactivationCandidateCellParameters" type="InterRatEsDeactivationCandidateCellParameters" minOccurs="0"/>

<element name="energySavingState" type="energySavingState" minOccurs="0"/>

<element name="isProbingCapable" type="isProbingCapable" minOccurs="0"/>

</all>

</complexType>

</element>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="IntraRatEsActivationOriginalCellLoadParameters">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="loadThreshold" type="loadThreshold" minOccurs="0"/>

<element name="timeDuration" type="timeDuration" minOccurs="0"/>

</all>

</complexType>

</element>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="IntraRatEsActivationCandidateCellsLoadParameters">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="loadThreshold" type="loadThreshold" minOccurs="0"/>

<element name="timeDuration" type="timeDuration" minOccurs="0"/>

</all>

</complexType>

</element>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="IntraRatEsDeactivationCandidateCellsLoadParameters">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="loadThreshold" type="loadThreshold" minOccurs="0"/>

<element name="timeDuration" type="timeDuration" minOccurs="0"/>

</all>

</complexType>

</element>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EsNotAllowedTimePeriod">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="startTimeandendTime" type="nn:startTimeandendTime" />

<element name="periodOfDay" type="nn:startTimeandendTime" />

<element name="daysOfWeekList" type="en:daysOfWeekList" />

<element name="listoftimeperiods" type="en:listoftimeperiods" />

</all>

</complexType>

</element>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="InterRatEsActivationOriginalCellParameters">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="loadThreshold" type="loadThreshold" minOccurs="0"/>

<element name="timeDuration" type="timeDuration" minOccurs="0"/>

</all>

</complexType>

</element>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="InterRatEsActivationCandidateCellParameters">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="loadThreshold" type="loadThreshold" minOccurs="0"/>

<element name="timeDuration" type="timeDuration" minOccurs="0"/>

</all>

</complexType>

</element>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="InterRatEsDeactivationCandidateCellParameters">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="loadThreshold" type="loadThreshold" minOccurs="0"/>

<element name="timeDuration" type="timeDuration" minOccurs="0"/>

</all>

</complexType>

</element>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="DRACHOptimizationFunction">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="ueAccProbilityDistPerSSBlist" type="UeAccProbilityDistPerSSBlist" minOccurs="0"/>

<element name="ueAccDelayProbilityDistPerSSBlist" type="UeAccDelayProbilityDistPerSSBlist" minOccurs="0"/>

<element name="drachOptimizationControl" type="boolean" minOccurs="0"/>

</all>

</complexType>

</element>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="DMROFunction">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="dmroControl" type=" boolean" minOccurs="0"/>

<element name="maximumDeviationHoTrigger" type="maximumDeviationHoTrigger" minOccurs="0"/>

<element name="minimumTimeBetweenHoTriggerChange" type="minimumTimeBetweenHoTriggerChange" minOccurs="0"/>

<element name="tstoreUEcntxt" type="tstoreUEcntxt" minOccurs="0"/>

</all>

</complexType>

</element>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="DPCIConfigurationFunction">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="nRPciList" type="NRPciList" minOccurs="0"/>

<element name="dPciConfigurationControl" type="boolean" minOccurs="0"/>

</all>

</complexType>

</element>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="CPCIConfigurationFunction">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="cSonPciList" type="CSonPciList" minOccurs="0"/>

<element name="cPciConfigurationControl" type="boolean" minOccurs="0"/>

</all>

</complexType>

</element>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="CESManagementFunction">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="cesSwitch" type="boolean" minOccurs="0"/>

<element name="energySavingState" type="energySavingState" minOccurs="0"/>

<element name="energySavingControl" type="energySavingControl" minOccurs="0"/>

</all>

</complexType>

</element>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

</schema>

Annex D (normative):
OpenAPI definition of the NR NRM

# D.1	General

This annex contains the OpenAPI definition of the NR NRM in YAML format.

The Information Service (IS) of the NR NRM is defined in clause 4.

Mapping rules to produce the OpenAPI definition based on the IS are defined in TS 32.160 [14].

# D.2	Void

# D.3	Void

# D.4	Solution Set (SS) definitions

## D.4.1	Void

## D.4.2	Void

## D.4.3	OpenAPI document "nrNrm.yaml"

openapi: 3.0.1

info:

title: NR NRM

version: 16.6.0

description: >-

OAS 3.0.1 specification of the NR NRM

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: TS 28.541 V16.6.0; 5G NRM, NR NRM

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.541/

paths: {}

components:

schemas:

#-------- Definition of types-----------------------------------------------------

GnbId:

type: string

GnbIdLength:

type: integer

minimum: 22

maximum: 32

GnbName:

type: string

maxLength: 150

GnbDuId:

type: number

minimum: 0

maximum: 68719476735

GnbCuUpId:

type: number

minimum: 0

maximum: 68719476735

Sst:

type: integer

maximum: 255

Snssai:

type: object

properties:

sst:

$ref: '#/components/schemas/Sst'

sd:

type: string

SnssaiList:

type: array

items:

$ref: '#/components/schemas/Snssai'

Mnc:

type: string

pattern: '[0-9]{3}|[0-9]{2}'

PlmnId:

type: object

properties:

mcc:

$ref: 'genericNrm.yaml#/components/schemas/Mcc'

mnc:

$ref: '#/components/schemas/Mnc'

PlmnIdList:

type: array

items:

$ref: '#/components/schemas/PlmnId'

PlmnInfo:

type: object

properties:

plmnId":

$ref: '#/components/schemas/PlmnId'

snssai:

$ref: '#/components/schemas/Snssai'

PlmnInfoList:

type: array

items:

$ref: '#/components/schemas/PlmnInfo'

GGnbId:

type: string

pattern: '^[0-9]{3}[0-9]{2,3}-(22|23|24|25|26|27|28|29|30|31|32)-[0-9]{1,10}'

GEnbId:

type: string

pattern: '^[0-9]{3}[0-9]{2,3}-(18|20|21|22)-[0-9]{1,7}'

GGnbIdList:

type: array

items: 
          $ref: '#/components/schemas/GGnbId'

GEnbIdList:

type: array

items: 
          $ref: '#/components/schemas/GEnbId'

NrPci:

type: integer

maximum: 503

NrTac:

type: integer

maximum: 16777215

Tai:

type: object

properties:

plmnId:

$ref: '#/components/schemas/PlmnId'

nrTac:

$ref: '#/components/schemas/NrTac'

BackhaulAddress:

type: object

properties:

gnbId:

$ref: '#/components/schemas/GnbId'

tai:

$ref: "#/components/schemas/Tai"

MappingSetIDBackhaulAddress:

type: object

properties:

setID:

type: integer

backhaulAddress:

$ref: '#/components/schemas/BackhaulAddress'

IntraRatEsActivationOriginalCellLoadParameters:

type: object

properties:

loadThreshold:

type: integer

timeDuration:

type: integer

IntraRatEsActivationCandidateCellsLoadParameters:

type: object

properties:

loadThreshold:

type: integer

timeDuration:

type: integer

IntraRatEsDeactivationCandidateCellsLoadParameters:

type: object

properties:

loadThreshold:

type: integer

timeDuration:

type: integer

EsNotAllowedTimePeriod:

type: object

properties:

startTimeandendTime:

type: string

periodOfDay:

type: string

daysOfWeekList:

type: string

listoftimeperiods:

type: string

InterRatEsActivationOriginalCellParameters:

type: object

properties:

loadThreshold:

type: integer

timeDuration:

type: integer

InterRatEsActivationCandidateCellParameters:

type: object

properties:

loadThreshold:

type: integer

timeDuration:

type: integer

InterRatEsDeactivationCandidateCellParameters:

type: object

properties:

loadThreshold:

type: integer

timeDuration:

type: integer

UeAccProbilityDistPerSSB:

type: object

properties:

targetProbability:

type: integer

numberofpreamblessent:

type: integer

UeAccDelayProbilityDistPerSSB:

type: object

properties:

targetProbability:

type: integer

accessdelay:

type: integer

NRPciList:

type: object

properties:

NRPci:

type: integer

CSonPciList:

type: object

properties:

NRPci:

type: integer

MaximumDeviationHoTrigger:

type: integer

minimum: -20

maximum: 20

MinimumTimeBetweenHoTriggerChange:

type: integer

minimum: 0

maximum: 604800

TstoreUEcntxt:

type: integer

minimum: 0

maximum: 1023

CellState:

type: string

enum:

- IDLE

- INACTIVE

- ACTIVE

CyclicPrefix:

type: string

enum:

- '15'

- '30'

- '60'

- '120'

TxDirection:

type: string

enum:

- DL

- UL

- DL and UL

BwpContext:

type: string

enum:

- DL

- UL

- SUL

IsInitialBwp:

type: string

enum:

- INITIAL

- OTHER

- SUL

QuotaType:

type: string

enum:

- STRICT

- FLOAT

IsESCoveredBy:

type: string

enum:

- NO

- PARTIAL

- FULL

RrmPolicyMember:

type: object

properties:

plmnId:

$ref: '#/components/schemas/PlmnId'

snssai:

$ref: '#/components/schemas/Snssai'

RrmPolicyMemberList:

type: array

items:

$ref: '#/components/schemas/RrmPolicyMember'

AddressWithVlan:

type: object

properties:

ipv4Address:

$ref: 'genericNrm.yaml#/components/schemas/Ipv4Addr'

ipv6Address:

$ref: 'genericNrm.yaml#/components/schemas/Ipv6Addr'

vlanId:

type: integer

minimum: 0

maximum: 4096

LocalAddress:

type: object

properties:

addressWithVlan:

$ref: '#/components/schemas/AddressWithVlan'

port:

type: integer

minimum: 0

maximum: 65535

RemoteAddress:

type: object

properties:

ipv4Address:

$ref: 'genericNrm.yaml#/components/schemas/Ipv4Addr'

ipv6Address:

$ref: 'genericNrm.yaml#/components/schemas/Ipv6Addr'

CellIndividualOffset:

type: object

properties:

rsrpOffsetSSB:

type: integer

rsrqOffsetSSB:

type: integer

sinrOffsetSSB:

type: integer

rsrpOffsetCSI-RS:

type: integer

rsrqOffsetCSI-RS:

type: integer

sinrOffsetCSI-RS:

type: integer

QOffsetRange:

type: integer

enum:

- -24

- -22

- -20

- -18

- -16

- -14

- -12

- -10

- -8

- -6

- -5

- -4

- -3

- -2

- -1

- 0

- 24

- 22

- 20

- 18

- 16

- 14

- 12

- 10

- 8

- 6

- 5

- 4

- 3

- 2

- 1

QOffsetRangeList:

type: object

properties:

rsrpOffsetSSB:

$ref: '#/components/schemas/QOffsetRange'

rsrqOffsetSSB:

$ref: '#/components/schemas/QOffsetRange'

sinrOffsetSSB:

$ref: '#/components/schemas/QOffsetRange'

rsrpOffsetCSI-RS:

$ref: '#/components/schemas/QOffsetRange'

rsrqOffsetCSI-RS:

$ref: '#/components/schemas/QOffsetRange'

sinrOffsetCSI-RS:

$ref: '#/components/schemas/QOffsetRange'

QOffsetFreq:

type: number

TReselectionNRSf:

type: integer

enum:

- 25

- 50

- 75

- 100

SsbPeriodicity:

type: integer

enum:

- 5

- 10

- 20

- 40

- 80

- 160

SsbDuration:

type: integer

enum:

- 1

- 2

- 3

- 4

- 5

SsbSubCarrierSpacing:

type: integer

enum:

- 15

- 30

- 120

- 240

CoverageShape:

type: integer

maximum: 65535

DigitalTilt:

type: integer

minimum: -900

maximum: 900

DigitalAzimuth:

type: integer

minimum: -1800

maximum: 1800

RSSetId:

type: integer

maximum: 4194303

RSSetType:

type: string

enum:

- RS1

- RS2

FrequencyDomainPara:

type: object

properties:

rimRSSubcarrierSpacing:

type: integer

rIMRSBandwidth:

type: integer

nrofGlobalRIMRSFrequencyCandidates:

type: integer

rimRSCommonCarrierReferencePoint:

type: integer

rimRSStartingFrequencyOffsetIdList:

type: array

items:

type: integer

SequenceDomainPara:

type: object

properties:

nrofRIMRSSequenceCandidatesofRS1:

type: integer

rimRSScrambleIdListofRS1:

type: array

items:

type: integer

nrofRIMRSSequenceCandidatesofRS2:

type: integer

rimRSScrambleIdListofRS2:

type: array

items:

type: integer

enableEnoughNotEnoughIndication:

type: string

enum:

- ENABLE

- DISABLE

RIMRSScrambleTimerMultiplier:

type: integer

RIMRSScrambleTimerOffset:

type: integer

TimeDomainPara:

type: object

properties:

dlULSwitchingPeriod1:

type: string

enum:

- MS0P5

- MS0P625

- MS1

- MS1P25

- MS2

- MS2P5

- MS3

- MS4

- MS5

- MS10

- MS20

symbolOffsetOfReferencePoint1:

type: integer

dlULSwitchingPeriod2:

type: string

enum:

- MS0P5

- MS0P625

- MS1

- MS1P25

- MS2

- MS2P5

- MS3

- MS4

- MS5

- MS10

- MS20

symbolOffsetOfReferencePoint2:

type: integer

totalnrofSetIdofRS1:

type: integer

totalnrofSetIdofRS2:

type: integer

nrofConsecutiveRIMRS1:

type: integer

nrofConsecutiveRIMRS2:

type: integer

consecutiveRIMRS1List:

type: array

items:

type: integer

consecutiveRIMRS2List:

type: array

items:

type: integer

enablenearfarIndicationRS1:

type: string

enum:

- ENABLE

- DISABLE

enablenearfarIndicationRS2:

type: string

enum:

- ENABLE

- DISABLE

RimRSReportInfo:

type: object

properties:

detectedSetID:

type: integer

propagationDelay:

type: integer

functionalityOfRIMRS:

type: string

enum:

- RS1

- RS2

- RS1forEnoughMitigation

- RS1forNotEnoughMitigation

RimRSReportConf:

type: object

properties:

reportIndicator:

type: string

enum:

- ENABLE

- DISABLE

reportInterval:

type: integer

nrofRIMRSReportInfo:

type: integer

maxPropagationDelay:

type: integer

rimRSReportInfoList:

type: array

items:

$ref: '#/components/schemas/RimRSReportInfo'

TceMappingInfo:

type: object

properties:

TceIPAddress:

oneOf:

- $ref: 'genericNrm.yaml#/components/schemas/Ipv4Addr'

- $ref: 'genericNrm.yaml#/components/schemas/Ipv6Addr'

TceID:

type: integer

PlmnTarget:

$ref: '#/components/schemas/PlmnId'

TceMappingInfoList:

type: array

items:

$ref: '#/components/schemas/TceMappingInfo'

#-------- Definition of abstract IOCs --------------------------------------------

RrmPolicy_-Attr:

type: object

properties:

resourceType:

type: string

rRMPolicyMemberList:

$ref: '#/components/schemas/RrmPolicyMemberList'

#-------- Definition of concrete IOCs --------------------------------------------

SubNetwork-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

$ref: 'genericNrm.yaml#/components/schemas/SubNetwork-Attr'

- $ref: 'genericNrm.yaml#/components/schemas/SubNetwork-ncO'

- type: object

properties:

SubNetwork:

$ref: '#/components/schemas/SubNetwork-Multiple'

ManagedElement:

$ref: '#/components/schemas/ManagedElement-Multiple'

NRFrequency:

$ref: '#/components/schemas/NRFrequency-Multiple'

ExternalGnbCuCpFunction:

$ref: '#/components/schemas/ExternalGnbCuCpFunction-Multiple'

ExternalENBFunction:

$ref: '#/components/schemas/ExternalENBFunction-Multiple'

EUtranFrequency:

$ref: '#/components/schemas/EUtranFrequency-Multiple'

DESManagementFunction:

$ref: '#/components/schemas/DESManagementFunction-Single'

DRACHOptimizationFunction:

$ref: '#/components/schemas/DRACHOptimizationFunction-Single'

DMROFunction:

$ref: '#/components/schemas/DMROFunction-Single'

DPCIConfigurationFunction:

$ref: '#/components/schemas/DPCIConfigurationFunction-Single'

CPCIConfigurationFunction:

$ref: '#/components/schemas/CPCIConfigurationFunction-Single'

CESManagementFunction:

$ref: '#/components/schemas/CESManagementFunction-Single'

Configurable5QISet:

$ref: '5gcNrm.yaml#/components/schemas/Configurable5QISet-Multiple'

RimRSGlobal:

$ref: '#/components/schemas/RimRSGlobal-Single'

Dynamic5QISet:

$ref: '5gcNrm.yaml#/components/schemas/Dynamic5QISet-Multiple'

ManagedElement-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

$ref: 'genericNRM.yaml#/components/schemas/ManagedElement-Attr'

- $ref: 'genericNRM.yaml#/components/schemas/ManagedElement-ncO'

- type: object

properties:

GnbDuFunction:

$ref: '#/components/schemas/GnbDuFunction-Multiple'

GnbCuUpFunction:

$ref: '#/components/schemas/GnbCuUpFunction-Multiple'

GnbCuCpFunction:

$ref: '#/components/schemas/GnbCuCpFunction-Multiple'

DESManagementFunction:

$ref: '#/components/schemas/DESManagementFunction-Single'

DRACHOptimizationFunction:

$ref: '#/components/schemas/DRACHOptimizationFunction-Single'

DMROFunction:

$ref: '#/components/schemas/DMROFunction-Single'

DPCIConfigurationFunction:

$ref: '#/components/schemas/DPCIConfigurationFunction-Single'

CPCIConfigurationFunction:

$ref: '#/components/schemas/CPCIConfigurationFunction-Single'

CESManagementFunction:

$ref: '#/components/schemas/CESManagementFunction-Single'

Configurable5QISet:

$ref: '5gcNrm.yaml#/components/schemas/Configurable5QISet-Multiple'

Dynamic5QISet:

$ref: '5gcNrm.yaml#/components/schemas/Dynamic5QISet-Multiple'

GnbDuFunction-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

gnbDuId:

$ref: '#/components/schemas/GnbDuId'

gnbDuName:

$ref: '#/components/schemas/GnbName'

gnbId:

$ref: '#/components/schemas/GnbId'

gnbIdLength:

$ref: '#/components/schemas/GnbIdLength'

rimRSReportConf:

$ref: '#/components/schemas/RimRSReportConf'

- $ref: 'genericNRM.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

RRMPolicyRatio:

$ref: '#/components/schemas/RRMPolicyRatio-Multiple'

NrCellDu:

$ref: '#/components/schemas/NrCellDu-Multiple'

Bwp-Multiple:

$ref: '#/components/schemas/Bwp-Multiple'

NrSectorCarrier-Multiple:

$ref: '#/components/schemas/NrSectorCarrier-Multiple'

EP_F1C:

$ref: '#/components/schemas/EP_F1C-Single'

EP_F1U:

$ref: '#/components/schemas/EP_F1U-Multiple'

GnbCuUpFunction-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

gnbId:

$ref: '#/components/schemas/GnbId'

gnbIdLength:

$ref: '#/components/schemas/GnbIdLength'

gnbCuUpId:

$ref: '#/components/schemas/GnbCuUpId'

plmnInfoList:

$ref: '#/components/schemas/PlmnInfoList'

configurable5QISetRef:

$ref: 'genericNRM.yaml#/components/schemas/Dn'

dynamic5QISetRef:

$ref: 'genericNRM.yaml#/components/schemas/Dn'

- $ref: 'genericNRM.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

RRMPolicyRatio:

$ref: '#/components/schemas/RRMPolicyRatio-Multiple'

EP_E1:

$ref: '#/components/schemas/EP_E1-Single'

EP_XnU:

$ref: '#/components/schemas/EP_XnU-Multiple'

EP_F1U:

$ref: '#/components/schemas/EP_F1U-Multiple'

EP_NgU:

$ref: '#/components/schemas/EP_NgU-Multiple'

EP_X2U:

$ref: '#/components/schemas/EP_X2U-Multiple'

EP_S1U:

$ref: '#/components/schemas/EP_S1U-Multiple'

GnbCuCpFunction-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

gnbId:

$ref: '#/components/schemas/GnbId'

gnbIdLength:

$ref: '#/components/schemas/GnbIdLength'

gnbCuName:

$ref: '#/components/schemas/GnbName'

plmnId:

$ref: '#/components/schemas/PlmnId'

x2BlackList:

$ref: '#/components/schemas/GGnbIdList'

xnBlackList:

$ref: '#/components/schemas/GGnbIdList'

x2WhiteList:

$ref: '#/components/schemas/GGnbIdList'

xnWhiteList:

$ref: '#/components/schemas/GGnbIdList'

xnHOBlackList:

$ref: '#/components/schemas/GGnbIdList'

x2HOBlackList:

$ref: '#/components/schemas/GEnbIdList'

mappingSetIDBackhaulAddress:

$ref: '#/components/schemas/MappingSetIDBackhaulAddress'

tceMappingInfoList:

$ref: '#/components/schemas/TceMappingInfoList'

configurable5QISetRef:

$ref: 'genericNRM.yaml#/components/schemas/Dn'

dynamic5QISetRef:

$ref: 'genericNRM.yaml#/components/schemas/Dn'

- $ref: 'genericNRM.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

RRMPolicyRatio:

$ref: '#/components/schemas/RRMPolicyRatio-Multiple'

NrCellCu:

$ref: '#/components/schemas/NrCellCu-Multiple'

EP_XnC:

$ref: '#/components/schemas/EP_XnC-Multiple'

EP_E1:

$ref: '#/components/schemas/EP_E1-Multiple'

EP_F1C:

$ref: '#/components/schemas/EP_F1C-Multiple'

EP_NgC:

$ref: '#/components/schemas/EP_NgC-Multiple'

EP_X2C:

$ref: '#/components/schemas/EP_X2C-Multiple'

DANRManagementFunction:

$ref: '#/components/schemas/DANRManagementFunction-Single'

DESManagementFunction:

$ref: '#/components/schemas/DESManagementFunction-Single'

DRACHOptimizationFunction:

$ref: '#/components/schemas/DRACHOptimizationFunction-Single'

DMROFunction:

$ref: '#/components/schemas/DMROFunction-Single'

NrCellCu-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

cellLocalId:

type: integer

plmnInfoList:

$ref: '#/components/schemas/PlmnInfoList'

nRFrequencyRef:

$ref: 'genericNRM.yaml#/components/schemas/Dn'

- $ref: 'genericNRM.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

RRMPolicyRatio:

$ref: '#/components/schemas/RRMPolicyRatio-Multiple'

NRCellRelation:

$ref: '#/components/schemas/NRCellRelation-Multiple'

EUtranCellRelation:

$ref: '#/components/schemas/EUtranCellRelation-Multiple'

NRFreqRelation:

$ref: '#/components/schemas/NRFreqRelation-Multiple'

EUtranFreqRelation:

$ref: '#/components/schemas/EUtranFreqRelation-Multiple'

DESManagementFunction:

$ref: '#/components/schemas/DESManagementFunction-Single'

DRACHOptimizationFunction:

$ref: '#/components/schemas/DRACHOptimizationFunction-Single'

DMROFunction:

$ref: '#/components/schemas/DMROFunction-Single'

CESManagementFunction:

$ref: '#/components/schemas/CESManagementFunction-Single'

NrCellDu-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

administrativeState:

$ref: 'genericNRM.yaml#/components/schemas/AdministrativeState'

operationalState:

$ref: 'genericNRM.yaml#/components/schemas/OperationalState'

cellLocalId:

type: integer

cellState:

$ref: '#/components/schemas/CellState'

plmnInfoList:

$ref: '#/components/schemas/PlmnInfoList'

nrPci:

$ref: '#/components/schemas/NrPci'

nrTac:

$ref: '#/components/schemas/NrTac'

arfcnDL:

type: integer

arfcnUL:

type: integer

arfcnSUL:

type: integer

bSChannelBwDL:

type: integer

bSChannelBwUL:

type: integer

bSChannelBwSUL:

type: integer

ssbFrequency:

type: integer

minimum: 0

maximum: 3279165

ssbPeriodicity:

$ref: '#/components/schemas/SsbPeriodicity'

ssbSubCarrierSpacing:

$ref: '#/components/schemas/SsbSubCarrierSpacing'

ssbOffset:

type: integer

minimum: 0

maximum: 159

ssbDuration:

$ref: '#/components/schemas/SsbDuration'

nrSectorCarrierRef:

type: array

items:

$ref: 'genericNRM.yaml#/components/schemas/Dn'

bwpRef:

type: array

items:

$ref: 'genericNRM.yaml#/components/schemas/Dn'

nRFrequencyRef:

$ref: 'genericNRM.yaml#/components/schemas/Dn'

victimSetRef:

$ref: 'genericNRM.yaml#/components/schemas/Dn'

aggressorSetRef:

$ref: 'genericNRM.yaml#/components/schemas/Dn'

- $ref: 'genericNRM.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

RRMPolicyRatio:

$ref: '#/components/schemas/RRMPolicyRatio-Multiple'

DPCIConfigurationFunction:

$ref: '#/components/schemas/DPCIConfigurationFunction-Single'

CPCIConfigurationFunction:

$ref: '#/components/schemas/CPCIConfigurationFunction-Single'

NRFrequency-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

type: object

properties:

absoluteFrequencySSB:

type: integer

minimum: 0

maximum: 3279165

ssbSubCarrierSpacing:

$ref: '#/components/schemas/SsbSubCarrierSpacing'

multiFrequencyBandListNR:

type: integer

minimum: 1

maximum: 256

EUtranFrequency-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

type: object

properties:

earfcnDL:

type: integer

minimum: 0

maximum: 262143

multiBandInfoListEutra:

type: integer

minimum: 1

maximum: 256

NrSectorCarrier-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

txDirection:

$ref: '#/components/schemas/TxDirection'

configuredMaxTxPower:

type: integer

arfcnDL:

type: integer

arfcnUL:

type: integer

bSChannelBwDL:

type: integer

bSChannelBwUL:

type: integer

sectorEquipmentFunctionRef:

$ref: 'genericNRM.yaml#/components/schemas/Dn'

- $ref: 'genericNRM.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

CommonBeamformingFunction:

$ref: '#/components/schemas/CommonBeamformingFunction-Single'

Bwp-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

bwpContext:

$ref: '#/components/schemas/BwpContext'

isInitialBwp:

$ref: '#/components/schemas/IsInitialBwp'

subCarrierSpacing:

type: integer

cyclicPrefix:

$ref: '#/components/schemas/CyclicPrefix'

startRB:

type: integer

numberOfRBs:

type: integer

- $ref: 'genericNRM.yaml#/components/schemas/ManagedFunction-ncO'

CommonBeamformingFunction-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

coverageShape:

$ref: '#/components/schemas/CoverageShape'

digitalAzimuth:

$ref: '#/components/schemas/DigitalAzimuth'

digitalTilt:

$ref: '#/components/schemas/DigitalTilt'

- type: object

properties:

Beam:

$ref: '#/components/schemas/Beam-Multiple'

Beam-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

beamIndex:

type: integer

beamType:

type: string

enum:

- SSB-BEAM

beamAzimuth:

type: integer

minimum: -1800

maximum: 1800

beamTilt:

type: integer

minimum: -900

maximum: 900

beamHorizWidth:

type: integer

minimum: 0

maximum: 3599

beamVertWidth:

type: integer

minimum: 0

maximum: 1800

RRMPolicyRatio-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: '#/components/schemas/RrmPolicy_-Attr'

- type: object

properties:

rRMPolicyMaxRatio:

type: integer

rRMPolicyMinRatio:

type: integer

rRMPolicyDedicatedRatio:

type: integer

NRCellRelation-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

type: object

properties:

nRTCI:

type: integer

cellIndividualOffset:

$ref: '#/components/schemas/CellIndividualOffset'

adjacentNRCellRef:

$ref: 'genericNRM.yaml#/components/schemas/Dn'

nRFrequencyRef:

$ref: 'genericNRM.yaml#/components/schemas/Dn'

isRemoveAllowed:

type: boolean

isHOAllowed:

type: boolean

isESCoveredBy:

$ref: '#/components/schemas/IsESCoveredBy'

isENDCAllowed:

type: boolean

EUtranCellRelation-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

adjacentEUtranCellRef:

$ref: 'genericNRM.yaml#/components/schemas/Dn'

- $ref: 'genericNRM.yaml#/components/schemas/ManagedFunction-ncO'

NRFreqRelation-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

type: object

properties:

offsetMO:

$ref: '#/components/schemas/QOffsetRangeList'

blackListEntry:

type: array

items:

type: integer

minimum: 0

maximum: 1007

blackListEntryIdleMode:

type: integer

cellReselectionPriority:

type: integer

cellReselectionSubPriority:

type: number

minimum: 0.2

maximum: 0.8

multipleOf: 0.2

pMax:

type: integer

minimum: -30

maximum: 33

qOffsetFreq:

$ref: '#/components/schemas/QOffsetFreq'

qQualMin:

type: number

qRxLevMin:

type: integer

minimum: -140

maximum: -44

threshXHighP:

type: integer

minimum: 0

maximum: 62

threshXHighQ:

type: integer

minimum: 0

maximum: 31

threshXLowP:

type: integer

minimum: 0

maximum: 62

threshXLowQ:

type: integer

minimum: 0

maximum: 31

tReselectionNr:

type: integer

minimum: 0

maximum: 7

tReselectionNRSfHigh:

$ref: '#/components/schemas/TReselectionNRSf'

tReselectionNRSfMedium:

$ref: '#/components/schemas/TReselectionNRSf'

nRFrequencyRef:

$ref: 'genericNRM.yaml#/components/schemas/Dn'

EUtranFreqRelation-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

type: object

properties:

cellIndividualOffset:

$ref: '#/components/schemas/CellIndividualOffset'

blackListEntry:

type: array

items:

type: integer

minimum: 0

maximum: 1007

blackListEntryIdleMode:

type: integer

cellReselectionPriority:

type: integer

cellReselectionSubPriority:

type: number

minimum: 0.2

maximum: 0.8

multipleOf: 0.2

pMax:

type: integer

minimum: -30

maximum: 33

qOffsetFreq:

$ref: '#/components/schemas/QOffsetFreq'

qQualMin:

type: number

qRxLevMin:

type: integer

minimum: -140

maximum: -44

threshXHighP:

type: integer

minimum: 0

maximum: 62

threshXHighQ:

type: integer

minimum: 0

maximum: 31

threshXLowP:

type: integer

minimum: 0

maximum: 62

threshXLowQ:

type: integer

minimum: 0

maximum: 31

tReselectionEutran:

type: integer

minimum: 0

maximum: 7

tReselectionNRSfHigh:

$ref: '#/components/schemas/TReselectionNRSf'

tReselectionNRSfMedium:

$ref: '#/components/schemas/TReselectionNRSf'

eUTranFrequencyRef:

$ref: 'genericNRM.yaml#/components/schemas/Dn'

DANRManagementFunction-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

type: object

properties:

intrasystemANRManagementSwitch:

type: boolean

intersystemANRManagementSwitch:

type: boolean

DESManagementFunction-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

type: object

properties:

desSwitch:

type: boolean

intraRatEsActivationOriginalCellLoadParameters:

$ref: "#/components/schemas/IntraRatEsActivationOriginalCellLoadParameters"

intraRatEsActivationCandidateCellsLoadParameters:

$ref: "#/components/schemas/IntraRatEsActivationCandidateCellsLoadParameters"

intraRatEsDeactivationCandidateCellsLoadParameters:

$ref: "#/components/schemas/IntraRatEsDeactivationCandidateCellsLoadParameters"

esNotAllowedTimePeriod:

$ref: "#/components/schemas/EsNotAllowedTimePeriod"

interRatEsActivationOriginalCellParameters:

$ref: "#/components/schemas/IntraRatEsActivationOriginalCellLoadParameters"

interRatEsActivationCandidateCellParameters:

$ref: "#/components/schemas/IntraRatEsActivationOriginalCellLoadParameters"

interRatEsDeactivationCandidateCellParameters:

$ref: "#/components/schemas/IntraRatEsActivationOriginalCellLoadParameters"

isProbingCapable:

type: string

enum:

- yes

- no

energySavingState:

type: string

enum:

- isNotEnergySaving

- isEnergySaving

DRACHOptimizationFunction-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

type: object

properties:

drachOptimizationControl:

type: boolean

ueAccProbilityDistPerSSB:

$ref: "#/components/schemas/UeAccProbilityDistPerSSB"

ueAccDelayProbilityDistPerSSB:

$ref: "#/components/schemas/UeAccDelayProbilityDistPerSSB"

- $ref: 'genericNRM.yaml#/components/schemas/ManagedFunction-ncO'

DMROFunction-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

type: object

properties:

dmroControl:

type: boolean

maximumDeviationHoTrigger:

$ref: '#/components/schemas/MaximumDeviationHoTrigger'

minimumTimeBetweenHoTriggerChange:

$ref: '#/components/schemas/MinimumTimeBetweenHoTriggerChange'

tstoreUEcntxt:

$ref: '#/components/schemas/TstoreUEcntxt'

DPCIConfigurationFunction-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

type: object

properties:

dPciConfigurationControl:

type: boolean

nRPciList:

$ref: "#/components/schemas/NRPciList"

CPCIConfigurationFunction-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

type: object

properties:

cPciConfigurationControl:

type: boolean

cSonPciList:

$ref: "#/components/schemas/CSonPciList"

CESManagementFunction-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

type: object

properties:

cesSwitch:

type: boolean

energySavingControl:

type: string

enum:

- toBeEnergySaving

- toBeNotEnergySaving

energySavingState:

type: string

enum:

- isNotEnergySaving

- isEnergySaving

RimRSGlobal-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

type: object

properties:

frequencyDomainPara:

$ref: '#/components/schemas/FrequencyDomainPara'

sequenceDomainPara:

$ref: '#/components/schemas/SequenceDomainPara'

timeDomainPara:

$ref: '#/components/schemas/TimeDomainPara'

RimRSSet:

$ref: '#/components/schemas/RimRSSet-Multiple'

RimRSSet-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

type: object

properties:

setId:

$ref: '#/components/schemas/RSSetId'

setType:

$ref: '#/components/schemas/RSSetType'

rimRSMonitoringStartTime:

type: string

rimRSMonitoringStopTime:

type: string

rimRSMonitoringWindowDuration:

type: integer

rimRSMonitoringWindowStartingOffset:

type: integer

rimRSMonitoringWindowPeriodicity:

type: integer

rimRSMonitoringOccasionInterval:

type: integer

rimRSMonitoringOccasionStartingOffset:

type: integer

nRCellDURefs:

$ref: 'genericNRM.yaml#/components/schemas/DnList'

ExternalGnbDuFunction-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

gnbId:

$ref: '#/components/schemas/GnbId'

gnbIdLength:

$ref: '#/components/schemas/GnbIdLength'

- $ref: 'genericNRM.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

EP_F1C:

$ref: '#/components/schemas/EP_F1C-Multiple'

EP_F1U:

$ref: '#/components/schemas/EP_F1U-Multiple'

ExternalGnbCuUpFunction-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

gnbId:

$ref: '#/components/schemas/GnbId'

gnbIdLength:

$ref: '#/components/schemas/GnbIdLength'

- $ref: 'genericNRM.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

EP_E1:

$ref: '#/components/schemas/EP_E1-Multiple'

EP_F1U:

$ref: '#/components/schemas/EP_F1U-Multiple'

EP_XnU:

$ref: '#/components/schemas/EP_XnU-Multiple'

ExternalGnbCuCpFunction-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: >-

genericNRM.yaml#/components/schemas/ManagedFunction-Attr

- type: object

properties:

gnbId:

$ref: '#/components/schemas/GnbId'

gnbIdLength:

$ref: '#/components/schemas/GnbIdLength'

plmnId:

$ref: '#/components/schemas/PlmnId'

- $ref: 'genericNRM.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

ExternalNrCellCu:

$ref: '#/components/schemas/ExternalNrCellCu-Multiple'

EP_XnC:

$ref: '#/components/schemas/EP_XnC-Multiple'

EP_E1:

$ref: '#/components/schemas/EP_E1-Multiple'

EP_F1C:

$ref: '#/components/schemas/EP_F1C-Multiple'

ExternalNrCellCu-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

cellLocalId:

type: integer

nrPci:

$ref: '#/components/schemas/NrPci'

plmnIdList:

$ref: '#/components/schemas/PlmnIdList'

nRFrequencyRef:

$ref: 'genericNRM.yaml#/components/schemas/Dn'

- $ref: 'genericNRM.yaml#/components/schemas/ManagedFunction-ncO'

ExternalENBFunction-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

eNBId:

type: integer

- $ref: 'genericNRM.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

ExternalEUTranCell:

$ref: '#/components/schemas/ExternalEUTranCell-Multiple'

ExternalEUTranCell-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

EUtranFrequencyRef:

$ref: 'genericNRM.yaml#/components/schemas/Dn'

- $ref: 'genericNRM.yaml#/components/schemas/ManagedFunction-ncO'

EP_XnC-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: '#/components/schemas/LocalAddress'

remoteAddress:

$ref: '#/components/schemas/RemoteAddress'

EP_E1-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: '#/components/schemas/LocalAddress'

remoteAddress:

$ref: '#/components/schemas/RemoteAddress'

EP_F1C-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: '#/components/schemas/LocalAddress'

remoteAddress:

$ref: '#/components/schemas/RemoteAddress'

EP_NgC-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: '#/components/schemas/LocalAddress'

remoteAddress:

$ref: '#/components/schemas/RemoteAddress'

EP_X2C-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: '#/components/schemas/LocalAddress'

remoteAddress:

$ref: '#/components/schemas/RemoteAddress'

EP_XnU-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: '#/components/schemas/LocalAddress'

remoteAddress:

$ref: '#/components/schemas/RemoteAddress'

EP_F1U-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: '#/components/schemas/LocalAddress'

remoteAddress:

$ref: '#/components/schemas/RemoteAddress'

EP_NgU-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: '#/components/schemas/LocalAddress'

remoteAddress:

$ref: '#/components/schemas/RemoteAddress'

epTransportRefs:

$ref: 'genericNrm.yaml#/components/schemas/DnList'

EP_X2U-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: '#/components/schemas/LocalAddress'

remoteAddress:

$ref: '#/components/schemas/RemoteAddress'

EP_S1U-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: '#/components/schemas/LocalAddress'

remoteAddress:

$ref: '#/components/schemas/RemoteAddress'

#-------- Definition of JSON arrays for name-contained IOCs ----------------------

SubNetwork-Multiple:

type: array

items:

$ref: '#/components/schemas/SubNetwork-Single'

ManagedElement-Multiple:

type: array

items:

$ref: '#/components/schemas/ManagedElement-Single'

GnbDuFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/GnbDuFunction-Single'

GnbCuUpFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/GnbCuUpFunction-Single'

GnbCuCpFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/GnbCuCpFunction-Single'

NrCellDu-Multiple:

type: array

items:

$ref: '#/components/schemas/NrCellDu-Single'

NrCellCu-Multiple:

type: array

items:

$ref: '#/components/schemas/NrCellCu-Single'

NRFrequency-Multiple:

type: array

minItems: 1

items:

$ref: '#/components/schemas/NRFrequency-Single'

EUtranFrequency-Multiple:

type: array

minItems: 1

items:

$ref: '#/components/schemas/EUtranFrequency-Single'

NrSectorCarrier-Multiple:

type: array

items:

$ref: '#/components/schemas/NrSectorCarrier-Single'

Bwp-Multiple:

type: array

items:

$ref: '#/components/schemas/Bwp-Single'

Beam-Multiple:

type: array

items:

$ref: '#/components/schemas/Beam-Single'

RRMPolicyRatio-Multiple:

type: array

items:

$ref: '#/components/schemas/RRMPolicyRatio-Single'

NRCellRelation-Multiple:

type: array

items:

$ref: '#/components/schemas/NRCellRelation-Single'

EUtranCellRelation-Multiple:

type: array

items:

$ref: '#/components/schemas/EUtranCellRelation-Single'

NRFreqRelation-Multiple:

type: array

items:

$ref: '#/components/schemas/NRFreqRelation-Single'

EUtranFreqRelation-Multiple:

type: array

items:

$ref: '#/components/schemas/EUtranFreqRelation-Single'

RimRSSet-Multiple:

type: array

items:

$ref: '#/components/schemas/RimRSSet-Single'

ExternalGnbDuFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/ExternalGnbDuFunction-Single'

ExternalGnbCuUpFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/ExternalGnbCuUpFunction-Single'

ExternalGnbCuCpFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/ExternalGnbCuCpFunction-Single'

ExternalNrCellCu-Multiple:

type: array

items:

$ref: '#/components/schemas/ExternalNrCellCu-Single'

ExternalENBFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/ExternalENBFunction-Single'

ExternalEUTranCell-Multiple:

type: array

items:

$ref: '#/components/schemas/ExternalEUTranCell-Single'

EP_E1-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_E1-Single'

EP_XnC-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_XnC-Single'

EP_F1C-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_F1C-Single'

EP_NgC-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_NgC-Single'

EP_X2C-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_X2C-Single'

EP_XnU-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_XnU-Single'

EP_F1U-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_F1U-Single'

EP_NgU-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_NgU-Single'

EP_X2U-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_X2U-Single'

EP_S1U-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_S1U-Single'

#-------- Definitions in TS 28.541 for TS 28.532 ---------------------------------

resources-nrNrm:

oneOf:

- $ref: '#/components/schemas/SubNetwork-Single'

- $ref: '#/components/schemas/ManagedElement-Single'

- $ref: '#/components/schemas/GnbDuFunction-Single'

- $ref: '#/components/schemas/GnbCuUpFunction-Single'

- $ref: '#/components/schemas/GnbCuCpFunction-Single'

- $ref: '#/components/schemas/NrCellCu-Single'

- $ref: '#/components/schemas/NrCellDu-Single'

- $ref: '#/components/schemas/NRFrequency-Single'

- $ref: '#/components/schemas/EUtranFrequency-Single'

- $ref: '#/components/schemas/NrSectorCarrier-Single'

- $ref: '#/components/schemas/Bwp-Single'

- $ref: '#/components/schemas/CommonBeamformingFunction-Single'

- $ref: '#/components/schemas/Beam-Single'

- $ref: '#/components/schemas/RRMPolicyRatio-Single'

- $ref: '#/components/schemas/NRCellRelation-Single'

- $ref: '#/components/schemas/EUtranCellRelation-Single'

- $ref: '#/components/schemas/NRFreqRelation-Single'

- $ref: '#/components/schemas/EUtranFreqRelation-Single'

- $ref: '#/components/schemas/DANRManagementFunction-Single'

- $ref: '#/components/schemas/DESManagementFunction-Single'

- $ref: '#/components/schemas/DRACHOptimizationFunction-Single'

- $ref: '#/components/schemas/DMROFunction-Single'

- $ref: '#/components/schemas/DPCIConfigurationFunction-Single'

- $ref: '#/components/schemas/CPCIConfigurationFunction-Single'

- $ref: '#/components/schemas/CESManagementFunction-Single'

- $ref: '#/components/schemas/RimRSGlobal-Single'

- $ref: '#/components/schemas/RimRSSet-Single'

- $ref: '#/components/schemas/ExternalGnbDuFunction-Single'

- $ref: '#/components/schemas/ExternalGnbCuUpFunction-Single'

- $ref: '#/components/schemas/ExternalGnbCuCpFunction-Single'

- $ref: '#/components/schemas/ExternalNrCellCu-Single'

- $ref: '#/components/schemas/ExternalENBFunction-Single'

- $ref: '#/components/schemas/ExternalEUTranCell-Single'

- $ref: '#/components/schemas/EP_XnC-Single'

- $ref: '#/components/schemas/EP_E1-Single'

- $ref: '#/components/schemas/EP_F1C-Single'

- $ref: '#/components/schemas/EP_NgC-Single'

- $ref: '#/components/schemas/EP_X2C-Single'

- $ref: '#/components/schemas/EP_XnU-Single'

- $ref: '#/components/schemas/EP_F1U-Single'

- $ref: '#/components/schemas/EP_NgU-Single'

- $ref: '#/components/schemas/EP_X2U-Single'

- $ref: '#/components/schemas/EP_S1U-Single'

Annex E (normative):
YANG definitions for NR NRM

# E.1	General

This annex contains the YANG definitions for the NR and NG-RAN NRM, in accordance with NR and NG-RAN NRM information model definitions specified in clause 4.

# E.2	Void

# E.3	Void

# E.4	Void

# E.5	Modules

## E.5.1	module _3gpp-nr-nrm-beam@2019-11-22.yang

module _3gpp-nr-nrm-beam {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-nrnetwork-beam";

prefix "beam3gpp";

import _3gpp-nr-nrm-commonbeamformingfunction { prefix cbeamff3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-nr-nrm-gnbdufunction { prefix gnbdu3gpp; }

import _3gpp-nr-nrm-nrsectorcarrier { prefix nrsectcarr3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the Beam Information

Object Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "TS 28.541 5G Network Resource Model (NRM)";

revision 2019-11-22 {

description "Initial revision";

reference "S5-197643";

}

typedef BeamType {

type enumeration {

enum SSB-BEAM;

}

}

grouping BeamGrp {

description "Represents the Beam IOC.";

reference "TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

leaf beamIndex {

description "Index of the beam. ";

mandatory true;

type int32;

}

leaf beamType {

description "The type of the beam. ";

mandatory false;

type BeamType;

}

leaf beamAzimuth {

description "The azimuth of a beam transmission, which means the horizontal beamforming pointing angle (beam peak direction) in the (Phi) φ-axis in 1/10th degree resolution.  The pointing angle is the direction equal to the geometric centre of the half-power contour of the beam relative to the reference plane. Zero degree implies explicit antenna bearing (boresight). Positive angle implies clockwise from the antenna bearing.";

reference "TS 38.104, TS 38.901, TS 28.662";

mandatory false;

type int32 { range "-1800..1800"; }

units "0.1";

}

leaf beamTilt {

description "The tilt of a beam transmission, which means the vertical beamforming pointing angle (beam peak direction) in the (Theta) θ-axis in 1/10th degree resolution.

The pointing angle is the direction equal to the geometric centre of the half-power contour of the beam relative to the reference plane. Positive value implies downtilt.";

reference "TS 38.104, TS 38.901, TS 28.662";

mandatory false;

type int32 { range "-900..900"; }

units "0.1";

}

leaf beamHorizWidth {

description " The Horizontal beamWidth of a beam transmission, which means the horizontal beamforming half-power (3dB down) beamwidth in the (Phi) φ-axis in 1/10th degree resolution.";

reference "TS 38.104, TS 38.901";

mandatory false;

type int32 { range "0..3599"; }

units "0.1";

}

leaf beamVertWidth {

description " The Vertical beamWidth of a beam transmission, which means the vertical beamforming half-power (3dB down) beamwidth in the (Theta) θ-axis in 1/10th degree resolution.";

reference "TS 38.104, TS 38.901";

mandatory false;

type int32 { range "0..1800"; }

units "0.1";

}

}

augment "/me3gpp:ManagedElement/gnbdu3gpp:GNBDUFunction/nrsectcarr3gpp:NRSectorCarrier/cbeamff3gpp:CommonBeamformingFunction" {

list Beam {

description "Represents the per-Beam information required for, e.g. beam performance management utilizing measurements generated in the RAN. Can have spatial attributes of horizontal/azimuth (ie: Phi φ-axis) and vertical/tilt (ie: Theta θ-axis) beam pointing direction and beam width attributes.";

reference "TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses BeamGrp;

}

}

}

}

## E.5.1a	module _3gpp-nr-nrm-bwp@2019-10-28.yang

module _3gpp-nr-nrm-bwp {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-bwp";

prefix "bwp3gpp";

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-gnbdufunction { prefix gnbdu3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the BWP Information Object Class

(IOC) that is part of the NR Network Resource Model (NRM).";

reference "TS 28.541 5G Network Resource Model (NRM)";

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 {

description "Initial revision";

}

typedef CyclicPrefix {

type enumeration {

enum NORMAL;

enum EXTENDED;

}

}

typedef BwpContext {

type enumeration {

enum DL;

enum UL;

enum SUL;

}

}

typedef IsInitialBwp {

type enumeration {

enum INITIAL;

enum OTHER;

}

}

grouping BWPGrp {

description "Represents the BWP IOC.";

reference "TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

leaf bwpContext {

description "Identifies whether the object is used for downlink, uplink

or supplementary uplink.";

mandatory true;

type BwpContext;

}

leaf isInitialBwp {

description "Identifies whether the object is used for initial or other

BWP.";

mandatory true;

type IsInitialBwp;

}

leaf subCarrierSpacing {

description "Subcarrier spacing configuration for a BWP.";

reference "TS 38.104";

mandatory true;

type uint32 { range "5 | 30 | 60 | 120"; }

units kHz;

}

leaf cyclicPrefix {

description "Cyclic prefix, which may be normal or extended.";

reference "TS 38.211";

mandatory true;

type CyclicPrefix;

}

leaf startRB {

description "Offset in common resource blocks to common resource block 0

for the applicable subcarrier spacing for a BWP.";

reference "N_BWP_start in TS 38.211";

mandatory true;

type uint32;

}

leaf numberOfRBs {

description "Number of physical resource blocks for a BWP.";

reference "N_BWP_size in TS 38.211";

mandatory true;

type uint32;

}

}

augment "/me3gpp:ManagedElement/gnbdu3gpp:GNBDUFunction" {

list BWP {

description "Represents a bandwidth part (BWP).";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses BWPGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## E.5.1b	module _3gpp-nr-nrm-commonbeamformingfunction@2019-11-22.yang

module _3gpp-nr-nrm-commonbeamformingfunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-nrnetwork-commonbeamformingfunction";

prefix "combeamformfunc3gpp";

import _3gpp-nr-nrm-nrsectorcarrier { prefix nrsectcarr3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-nr-nrm-gnbdufunction { prefix gnbdu3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the CommonBeamformingFuntion Information

Object Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "TS 28.541 5G Network Resource Model (NRM)";

revision 2019-11-22 {

description "Initial revision";

reference "S5-197643";

}

grouping CommonBeamformingFunctionGrp {

description "Represents the CommonBeamformingFunction IOC.";

reference "TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

leaf coverageShape {

description "Identifies the sector carrier coverage shape described by the envelope of the contained SSB beams. The coverage shape is implementation dependent.";

mandatory true;

type int32 { range "0..65535"; }

}

leaf digitalAzimuth {

description "Digitally-controlled azimuth through beamforming. It represents the horizontal pointing direction of the antenna relative to the antenna bore sight, representing the total non-mechanical horizontal pan of the selected coverageShape. Positive value gives azimuth to the right and negative value gives an azimuth to the left.";

reference "TS 38.104, TS 38.901, TS 28.662";

type int32 { range "-1800..1800"; }

units "0.1";

}

leaf digitalTilt {

description "Digitally-controlled tilt through beamforming. It represents the vertical pointing direction of the antenna relative to the antenna bore sight, representing the total non-mechanical vertical tilt of the selected coverageShape. Positive value gives downwards tilt and negative value gives upwards tilt.";

reference "TS 38.104, TS 38.901, TS 28.662";

type int32 { range "-900..900"; }

units "0.1";

}

}

augment "/me3gpp:ManagedElement/gnbdu3gpp:GNBDUFunction/nrsectcarr3gpp:NRSectorCarrier" {

list CommonBeamformingFunction {

description "Represents common beamforming functionality (eg: SSB beams) for the NRSectorCarrier.";

reference "TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses CommonBeamformingFunctionGrp;

}

}

}

}

## E.5.2	module_3gpp-nr-nrm-ep@.yang

module _3gpp-nr-nrm-ep {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-ep";

prefix "ep3gpp";

import _3gpp-common-ep-rp { prefix eprp3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-gnbcucpfunction { prefix gnbcucp3gpp; }

import _3gpp-nr-nrm-gnbcuupfunction { prefix gnbcuup3gpp; }

import _3gpp-nr-nrm-gnbdufunction { prefix gnbdu3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the NR related endpoint

Information Object Classes (IOCs) that are part of the NR Network

Resource Model (NRM).";

reference "TS 28.541 5G Network Resource Model (NRM)";

revision 2020-03-02 { reference S5-201191"; }

revision 2019-06-17 {

description "Initial revision";

}

grouping EP_E1Grp {

description "Represents the EP_E1 IOC.";

reference "TS 28.541, TS 38.401";

uses eprp3gpp:EP_Common;

}

grouping EP_F1CGrp {

description "Represents the EP_F1C IOC.";

reference "TS 28.541, TS 38.470";

uses eprp3gpp:EP_Common;

}

grouping EP_F1UGrp {

description "Represents the EP_F1U IOC.";

reference "TS 28.541, TS 38.470";

uses eprp3gpp:EP_Common;

}

grouping EP_XnCGrp {

description "Represents the EP_XnC IOC.";

reference "TS 28.541, TS 38.420";

uses eprp3gpp:EP_Common;

}

grouping EP_XnUGrp {

description "Represents the EP_XnU IOC.";

reference "TS 28.541, TS 38.420";

uses eprp3gpp:EP_Common;

}

grouping EP_NgCGrp {

description "Represents the EP_NgC IOC.";

reference "TS 28.541, TS 38.470";

uses eprp3gpp:EP_Common;

}

grouping EP_NgUGrp {

description "Represents the EP_NgU IOC.";

reference "TS 28.541, TS 38.470";

uses eprp3gpp:EP_Common;

}

grouping EP_X2CGrp {

description "Represents the EP_X2C IOC.";

reference "TS 28.541, TS 36.423";

uses eprp3gpp:EP_Common;

}

grouping EP_X2UGrp {

description "Represents the EP_X2U IOC.";

reference "TS 28.541, TS 36.425";

uses eprp3gpp:EP_Common;

}

grouping EP_S1UGrp {

description "Represents the EP_S1U IOC.";

reference "TS 28.541, TS 36.410";

uses eprp3gpp:EP_Common;

}

augment "/me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction" {

list EP_E1 {

description "Represents the local end point of the logical link,

supporting E1 interface between gNB-CU-CP and gNB-CU-UP.";

reference "TS 28.541, TS 38.401";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_E1Grp;

}

}

list EP_F1C {

description "Represents the local end point of the control plane

interface (F1-C) between the DU and CU or CU-CP.";

reference "TS 28.541, TS 38.470";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_F1CGrp;

}

}

list EP_NgC {

description "Represents the local end point of the control plane

interface (NG-C) between the gNB and NG-Core entity.";

reference "TS 28.541, TS 38.470";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_NgCGrp;

}

}

list EP_XnC {

description "Represents the local gNB node end point of the logical

link, supporting Xn application protocols, to a neighbour NG-RAN node

(including gNB and ng-eNB). The Xn Application PDUs are carried over

SCTP/IP/Data link layer/Physical layer stack.";

reference "TS 28.541, TS 38.420 clause 7";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_XnCGrp;

}

}

list EP_X2C {

description "Represents the local end point of the logical link,

supporting X2-C application protocols used in EN-DC, to a neighbour

eNB or en-gNB node.";

reference "TS 28.541, TS 36.423";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_X2CGrp;

}

}

}

augment "/me3gpp:ManagedElement/gnbcuup3gpp:GNBCUUPFunction" {

list EP_E1 {

description "Represents the local end point of the logical link,

supporting E1 interface between gNB-CU-CP and gNB-CU-UP.";

reference "TS 28.541, TS 38.401";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_E1Grp;

}

}

list EP_F1U {

description "Represents the local end point of the user plane

interface (F1-U) between the DU and CU or CU-UP.";

reference "TS 28.541, TS 38.470";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_F1UGrp;

}

}

list EP_NgU {

description "Represents the local end point of the NG user plane

(NG-U) interface between the gNB and the UPGW.";

reference "TS 28.541, TS 38.470";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_NgUGrp;

}

}

list EP_XnU {

description "Represents the one end-point of a logical link supporting

the Xn user plane (Xn-U) interface. The Xn-U interface provides

non-guaranteed delivery of user plane PDUs between two NG-RAN nodes.";

reference "TS 28.541, TS 38.420";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_XnUGrp;

}

}

list EP_X2U {

description "Represents the local end-point of a logical link supporting

the X2 user plane (X2-U) interface used in EN-DC.";

reference "TS 28.541, TS 36.425";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_X2UGrp;

}

}

list EP_S1U {

description "Represents the local end point of the logical link,

supporting S1-U interface towards a S-GW node.";

reference "TS 28.541, TS 36.410";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_S1UGrp;

}

}

}

augment "/me3gpp:ManagedElement/gnbdu3gpp:GNBDUFunction" {

list EP_F1C {

description "Represents the local end point of the control plane

interface (F1-C) between the DU and CU or CU-CP.";

reference "TS 28.541, TS 38.470";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_F1CGrp;

}

}

list EP_F1U {

description "Represents the local end point of the user plane

interface (F1-U) between the DU and CU or CU-UP.";

reference "TS 28.541, TS 38.470";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_F1UGrp;

}

}

}

}

## E.5.3	module _3gpp-nr-nrm-eutrancellrelation@2019-10-28.yang

module _3gpp-nr-nrm-eutrancellrelation {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-eutrancellrelation";

prefix "eutrancellrel3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-nr-nrm-gnbcucpfunction { prefix gnbcucp3gpp; }

import _3gpp-nr-nrm-nrcellcu { prefix nrcellcu3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the EUtranCellRelation Information

Object Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "TS 28.541 5G Network Resource Model (NRM)";

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 {

description "Initial revision";

}

typedef ActionAllowed {

type enumeration {

enum YES;

enum NO;

}

}

typedef EnergySavingCoverage {

type enumeration {

enum YES;

enum NO;

enum PARTIAL;

}

}

grouping EUtranCellRelationGrp {

description "Represents the EUtranCellRelation IOC.";

reference "TS 28.541, EUtranRelation in TS 28.658";

uses mf3gpp:ManagedFunctionGrp;

leaf tCI {

description "Target Cell Identifier. Consists of E-UTRAN Cell Global

Identifier (ECGI) and Physical Cell Identifier (PCI) of the target

cell. Identifies the target cell from the perspective of the parent

cell instance.";

mandatory true;

type uint64;

}

leaf isRemoveAllowed {

description "Indicates if the subject EUtranCellRelation can be removed

(deleted) or not. If YES, the subject EUtranCellRelation instance can

be removed (deleted). If NO, the subject EUtranCellRelation instance

shall not be removed (deleted) by any entity but an IRPManager.";

mandatory true;

type ActionAllowed;

}

leaf isHOAllowed {

description "Indicates if handover is allowed or prohibited. If YES,

handover is allowed from source cell to target cell. Source cell is

represented by the parent cell instance. Target cell is the adjacent

cell referenced by this EUtranCellRelation instance. If NO, handover

shall not be allowed.";

mandatory true;

type ActionAllowed;

}

leaf isENDCAllowed {

description "Indicates if EN-DC is allowed or prohibited. If TRUE,

the target cell is allowed to be used for EN-DC. The target cell is

referenced by the NRCellRelation that contains this isENDCAllowed.

If FALSE, EN-DC shall not be allowed.";

mandatory true;

type ActionAllowed;

}

leaf isICICInformationSendAllowed {

description "Indicates if ICIC (Inter Cell Interference Coordination)

load information message sending is allowed or prohibited. If YES,

ICIC load information message sending is allowed from source cell to

target cell. Source cell is represented by the parent cell instance.

Target cell is the adjacent cell referenced by this EUtranCellRelation

instance. If NO, ICIC load information message sending shall not be

allowed.";

reference "TS 36.423";

mandatory true;

type ActionAllowed;

}

leaf isLBAllowed {

description "Indicates if load balancing is allowed or prohibited from

source cell to target cell. If YES, load balancing is allowed from

source cell to target cell. Source cell is represented by the parent

cell instance. Target cell is the adjacent cell referenced by this

EUtranCellRelation instance. If NO, load balancing shall be prohibited

from source cell to target cell.";

mandatory true;

type ActionAllowed;

}

leaf isESCoveredBy {

description "Indicates whether the adjacent cell according to this

planning provides no, partial or full coverage for the parent cell

instance. Adjacent cells with this attribute equal to YES are

recommended to be considered as candidate cells to take over the

coverage when the original cell is about to be transferred to energy

saving state. The entirety of adjacent cells with this property equal

to PARTIAL are recommended to be considered as entirety of candidate

cells to take over the coverage when the original cell is about to be

transferred to energy saving state.";

mandatory true;

type EnergySavingCoverage;

}

leaf qOffset {

description "Offset applicable to a specific neighbouring cell used for

evaluating the cell as a candidate for cell re-selection. Corresponds

to parameter q-OffsetCell broadcast in SIB4 for intra-frequency cells

and in SIB5 for inter-frequency cells. Used for Mobility Robustness

Optimization.";

reference "TS 36.331";

mandatory true;

type types3gpp:QOffsetRange;

}

leaf cellIndividualOffset {

description "Offset applicable to a neighbouring cell. It is used for

evaluating the neighbouring cell for handover in connected mode. Used

by the HandOver parameter Optimization (HOO) function or Load

Balancing Optimization (LBO) function.";

reference "TS 36.331";

config false;

type types3gpp:QOffsetRange;

}

leaf adjacentCell {

description "Reference to an EUtranCellFDD/TDD or

ExternalEUtranCellFDD/TDD instance.";

mandatory true;

type types3gpp:DistinguishedName;

}

}

augment /me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction/nrcellcu3gpp:NRCellCU {

list EUtranCellRelation {

description "Represents a relation between an NR cell and an E-UTRAN cell.";

reference "TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EUtranCellRelationGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## E.5.4	module _3gpp-nr-nrm-eutranetwork@2019-06-17.yang

module _3gpp-nr-nrm-eutranetwork {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-eutranetwork";

prefix "eutranet3gpp";

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the EUtraNetwork Information Object

Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "TS 28.541 5G Network Resource Model (NRM)";

revision 2019-06-17 {

description "Initial revision";

}

feature ExternalsUnderEUtraNetwork {

description "Classes representing external entities like EUtranFrequency,

ExternalENBFunction are contained under a EUtraNetwork list/class.";

}

grouping EUtraNetworkGrp {

description "Represents the EUtraNetwork IOC.";

reference "TS 28.541";

uses subnet3gpp:SubNetworkGrp;

}

list EUtraNetwork {

description "A subnetwork containing gNB external E-UTRAN entities.";

reference "TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EUtraNetworkGrp;

leaf-list parents {

description "Reference to all containg EUtraNetwork instances

in strict order from the root EUtraNetwork down to the immediate

parent EUtraNetwork.

If EUtraNetworks form a containment hierarchy this is

modeled using references between the child EUtraNetwork and the parent

EUtraNetworks.

This reference MUST NOT be present for the top level EUtraNetwork and

MUST be present for other EUtraNetworks.";

type leafref {

path "../../../EUtraNetwork/id";

}

}

leaf-list containedChildren{

description "Reference to all directly contained EUtraNetwork instances.

If EUtraNetworks form a containment hierarchy this is

modeled using references between the child EUtraNetwork and the parent

EUtraNetwork.";

type leafref {

path "../../../EUtraNetwork/id";

}

}

}

}

}

## E.5.5	module _3gpp-nr-nrm-eutranfreqrelation@2019-10-28.yang

module _3gpp-nr-nrm-eutranfreqrelation {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-eutranfreqrelation";

prefix "eutranfreqrel3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-gnbcucpfunction { prefix gnbcucp3gpp; }

import _3gpp-nr-nrm-nrcellcu { prefix nrcellcu3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the EUtranFreqRelation Information

Object Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "TS 28.541 5G Network Resource Model (NRM)";

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 {

description "Initial revision";

}

grouping EUtranFreqRelationGrp {

description "Represents the EUtranFreqRelation IOC.";

reference "TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

leaf cellIndividualOffset {

description "Offset applicable to a neighbouring cell. Used for

evaluating the neighbouring cell for handover in connected mode.

Used by the HandOver parameter Optimization (HOO) function or

Load Balancing Optimization (LBO) function.";

reference "cellIndividualOffset in MeasObjectEUTRA in TS 38.331";

default 0;

type types3gpp:QOffsetRange;

}

leaf-list blackListEntry {

description "A list of Physical Cell Identities (PCIs) that are

blacklisted in E-UTRAN measurements.";

reference "TS 38.331";

min-elements 0;

type uint16 { range "0..1007"; }

}

leaf-list blackListEntryIdleMode {

description "A list of Physical Cell Identities (PCIs) that are

blacklisted in SIB4 and SIB5.";

min-elements 0;

type uint16 { range "0..1007"; }

}

leaf cellReselectionPriority {

description "The absolute priority of the carrier frequency used by the

cell reselection procedure. Value 0 means lowest priority. The value

must not already used by other RAT, i.e. equal priorities between RATs

are not supported. The UE behaviour when no value is entered is

specified in clause 5.2.4.1 of TS 38.304.";

reference "CellReselectionPriority in TS 38.331, priority in

TS 38.304";

mandatory true;

type int32 { range "0..7"; }

}

leaf cellReselectionSubPriority {

description "Indicates a fractional value to be added to the value of

cellReselectionPriority to obtain the absolute priority of the

concerned carrier frequency for E-UTRA and NR.";

reference "TS 38.331";

type uint8 { range "2 | 4 | 6 | 8"; }

units "0.1";

}

leaf pMax {

description "Used for calculation of the parameter Pcompensation

(defined in TS 38.304), at cell reselection to a cell.";

reference "PEMAX in TS 38.101-1";

mandatory true;

type int32 { range "-30..33"; }

units dBm;

}

leaf qOffsetFreq {

description "The frequency specific offset applied when evaluating

candidates for cell reselection.";

type int32;

default 0;

}

leaf qQualMin {

description "Indicates the minimum required quality level in the cell.

Value 0 means that it is not sent and UE applies in such case the

(default) value of negative infinity for Qqualmin. Sent in SIB3 or

SIB5.";

reference "qQualMin in TS 38.304";

mandatory true;

type int32 { range "-34..-3 | 0"; }

units dB;

}

leaf qRxLevMin {

description "Indicates the required minimum received Reference Symbol

Received Power (RSRP) level in the (E-UTRA) frequency for cell

reselection. Broadcast in SIB3 or SIB5, depending on whether the

related frequency is intra- or inter-frequency. Resolution is 2.";

reference "Qrxlevmin in TS 38.304";

mandatory true;

type int32 { range "-140..-44"; }

units dBm;

}

leaf threshXHighP {

description "Specifies the Srxlev threshold used by the UE when

reselecting towards a higher priority RAT/frequency than the current

serving frequency. Each frequency of NR and E-UTRAN might have a

specific threshold. Resolution is 2.";

reference "ThreshX, HighP in TS 38.304";

mandatory true;

type int32 { range "0..62"; }

units dB;

}

leaf threshXHighQ {

description "Specifies the Squal threshold used by the UE when

reselecting towards a higher priority RAT/frequency than the current

serving frequency. Each frequency of NR and E-UTRAN might have a

specific threshold.";

reference "ThreshX, HighQ in TS 38.304";

mandatory true;

type int32 { range 0..31; }

units dB;

}

leaf threshXLowP {

description "Specifies the Srxlev threshold used by the UE when

reselecting towards a lower priority RAT/frequency than the current

serving frequency. Each frequency of NR and E-UTRAN might have a

specific threshold. Resolution is 2.";

reference "ThreshX, LowP in TS 38.304";

mandatory true;

type int32 { range "0..62"; }

units dB;

}

leaf threshXLowQ {

description "Specifies the Squal threshold used by the UE when

reselecting towards a lower priority RAT/frequency than the current

serving frequency. Each frequency of NR and E-UTRAN might have a

specific threshold.";

reference "ThreshX, LowQ in TS 38.304";

mandatory false;

type int32 { range "0..31"; }

units dB;

}

leaf tReselectionEutra {

description "Cell reselection timer for intra frequency E-UTRA cell

reselection. May be used for Mobility Robustness Optimization.";

reference "t-ReselectionEUTRA in TS 36.331 and in TS 23.207";

mandatory true;

type uint8 { range "0..7"; }

units s;

}

leaf tReselectionEutraSfHigh {

description "The attribute tReselectionEutra (parameter TreselectionEUTRA

in TS 38.304) multiplied with this scaling factor if the UE is in

high mobility state.";

reference "Speed dependent ScalingFactor for TreselectionEUTRA for high

mobility state in TS 38.304";

mandatory true;

type uint8 { range "25 | 50 | 75 | 100"; }

units %;

}

leaf tReselectionEutraSfMedium {

description "The attribute tReselectionEutra (parameter TreselectionEUTRA

in TS 38.304) multiplied with this scaling factor if the UE is in

medium mobility state.";

reference "Speed dependent ScalingFactor for TreselectionEUTRA for medium

mobility state in TS 38.304";

mandatory true;

type uint8 { range "25 | 50 | 75 | 100"; }

units %;

}

leaf eUtranFrequencyRef {

description "Reference to a corresponding EUtranFrequency instance.";

mandatory true;

type types3gpp:DistinguishedName;

}

}

augment /me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction/nrcellcu3gpp:NRCellCU {

list EUtranFreqRelation {

description "Represents a frequency relation between an NR cell and an

E-UTRAN cell.";

reference "TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EUtranFreqRelationGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## E.5.6	module _3gpp-nr-nrm-eutranfrequency@2019-10-28.yang

module _3gpp-nr-nrm-eutranfrequency {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-eutranfrequency";

prefix "eutraneteutranfreq3gpp";

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-nr-nrm-eutranetwork { prefix eutranet3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the EUtranFrequency Information

Object Class (IOC), that is part of the NR Network Resource Model (NRM).";

reference "TS 28.541 5G Network Resource Model (NRM),

TS 28.658 (E-UTRAN) Network Resource Model (NRM)";

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 {

description "Initial revision";

}

grouping EUtranFrequencyGrp {

description "Represents the EUtranFrequency IOC.";

reference "TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

leaf earfcnDL {

description "Specifies the channel number for the central DL frequency.";

reference "TS 36.101";

mandatory true;

type uint32 { range "0..262143"; }

}

leaf-list multiBandInfoListEutra {

description "List of additional frequency bands the frequency belongs to.";

config false;

min-elements 0;

type uint16 { range "1..256"; }

}

}

grouping EUtranFrequencyWrapper {

list EUtranFrequency {

description "Represents certain E-UTRAN frequency properties.";

reference "TS 28.658";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EUtranFrequencyGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

augment "/subnet3gpp:SubNetwork" {

if-feature subnet3gpp:ExternalsUnderSubNetwork ;

uses EUtranFrequencyWrapper ;

}

augment "/eutranet3gpp:EUtraNetwork" {

if-feature eutranet3gpp:ExternalsUnderEUtraNetwork;

uses EUtranFrequencyWrapper ;

}

}

## E.5.7	module _3gpp-nr-nrm-externalamffunction@2019-10-28.yang

module _3gpp-nr-nrm-externalamffunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-externalamffunction";

prefix "extamf3gpp";

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-nr-nrm-nrnetwork { prefix nrnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the ExternalAMFFunction Information

Object Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "TS 28.541 5G Network Resource Model (NRM)";

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 {

description "Initial revision";

}

grouping ExternalAMFFunctionGrp {

description "Represents the ExternalAMFFunction IOC.";

reference "TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least

one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a

Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

container aMFIdentifier {

presence true;

description "An AMF identifier, comprising an AMF Region ID, an AMF Set ID and an AMF Pointer.";

uses types3gpp:AmfIdentifier;

}

}

grouping ExternalAMFFunctionWrapper {

list ExternalAMFFunction {

description "Represents the properties, known by the management

function, of a AMFFunction managed by another management

function.";

reference "TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalAMFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

augment "/subnet3gpp:SubNetwork" {

if-feature subnet3gpp:ExternalsUnderSubNetwork ;

uses ExternalAMFFunctionWrapper;

}

augment "/nrnet3gpp:NRNetwork" {

if-feature nrnet3gpp:ExternalsUnderNRNetwork;

uses ExternalAMFFunctionWrapper;

}

}

## E.5.8	module _3gpp-nr-nrm-externalenbfunction@2019-10-28.yang

module _3gpp-nr-nrm-externalenbfunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-externalenbfunction";

prefix "extenb3gpp";

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-nr-nrm-eutranetwork { prefix eutranet3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the ExternalENBFunction

Information Object Class (IOC) that is part of the NR Network Resource

Model (NRM).";

reference "TS 28.541 5G Network Resource Model (NRM),

TS 28.658 (E-UTRAN) Network Resource Model (NRM)";

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 {

description "Initial revision";

}

grouping ExternalENBFunctionGrp {

description "Represets the ExternalENBFunction IOC.";

reference "TS 28.658";

uses mf3gpp:ManagedFunctionGrp;

leaf eNBId {

description "Unambiguously identifies an eNodeB within a PLMN.";

reference "TS 36.413, TS 36.300";

mandatory true;

type int32 { range "0..268435455"; } // Representing 28 bit eNB ID.

// 18, 20 and 21 bit eNB IDs also

// allowed.

}

}

grouping ExternalENBFunctionWrapper {

list ExternalENBFunction {

description "Represents an external eNB functionality.";

reference "TS 28.658";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalENBFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

augment "/subnet3gpp:SubNetwork" {

if-feature subnet3gpp:ExternalsUnderSubNetwork ;

uses ExternalENBFunctionWrapper;

}

augment "/eutranet3gpp:EUtraNetwork" {

if-feature eutranet3gpp:ExternalsUnderEUtraNetwork;

uses ExternalENBFunctionWrapper;

}

}

## E.5.9	module_3gpp-nr-nrm-externaleutrancell@2019-10-28.yang

module _3gpp-nr-nrm-externaleutrancell {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-externaleutrancell";

prefix "exteutrancell3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-nr-nrm-eutranetwork { prefix eutranet3gpp; }

import _3gpp-nr-nrm-externalenbfunction { prefix extenb3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the ExternalEUtranCellFDD and

ExternalEUtranCellTDD Information Object Classes (IOCs) that are part

of the NR Network Resource Model (NRM).";

reference "TS 28.541 5G Network Resource Model (NRM),

TS 28.658 (E-UTRAN) Network Resource Model (NRM)";

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 {

description "Initial revision";

}

grouping ExternalEUtranGenericCellGrp {

description "Represents the ExternalEUtranGenericCell IOC.";

reference "TS 28.658";

uses mf3gpp:ManagedFunctionGrp;

leaf pci {

description "The Physical Cell Identity (PCI) of the cell (for

NM-Centralized, EM-Centralized and Distributed PCI assignment cases).

In the case of NM-Centralized PCI assignment, see TS 36.300.";

reference "TS 36.211";

mandatory true;

type int32 { range "0..503"; }

}

list plmnIdList {

description "List of unique identities for PLMNs. A cell can broadcast

up to 6 PLMN IDs. This is to support the case that one cell can be

used by up to 6 operator's core networks. The PLMN(s) included in this

list will use the same single tracking area code (TAC) and the same

Cell Identity (cellLocalId) for sharing the radio access network

resources. One member of plmnIdList is the primary PLMN ID. A PLMN ID

included in this list cannot be included in the cellAccessInfoList.

The PLMN ID is composed of a Mobile Country Code (MCC) and a Mobile

Network Code (MNC).";

reference "TS 36.300, TS 36.331, TS 23.003";

key "mcc mnc";

min-elements 1;

max-elements 6;

uses types3gpp:PLMNId;

}

leaf cellLocalId {

description "Unambiguously identifies a cell within an eNodeB.";

reference "NCI defined in TS 38.300";

type int32 {range "0..255"; }

}

leaf eNBId {

description "Unambiguously identifies an eNodeB within a PLMN.";

reference "TS 36.413, TS 36.300";

mandatory true;

type int32 { range "0..268435455"; } // Representing 28 bit eNB ID.

// 18, 20 and 21 bit eNB IDs also

// allowed.

}

}

grouping ExternalEUtranCellFDDGrp {

description "Represents the ExternalEUtranCellFDD IOC.";

reference "TS 28.658";

uses ExternalEUtranGenericCellGrp;

leaf earfcnDL {

description "The channel number for the central DL frequency.";

reference "TS 36.101";

mandatory true;

type int32 { range "0..17999 | 46590..262143"; }

}

leaf earfcnUL {

description "The channel number for the central UL frequency. Value 0

means that the UL channel number is N/A for the DL-only bands.";

reference "TS 36.101";

mandatory true;

type int32 { range "0 | 18000..35999 | 46590..262143"; }

}

}

grouping ExternalEUtranCellTDDGrp {

description "Represents the ExternalEUtranCellTDD IOC.";

reference "TS 28.658";

uses ExternalEUtranGenericCellGrp;

leaf earfcn {

description "The frequency number for the central frequency.";

reference "TS 36.104";

mandatory true;

type int32 { range "36000..262143"; }

}

}

grouping ExternalEUtranCellFDDWrapper {

list ExternalEUtranCellFDD {

description "Represents the common properties of external E-UTRAN FDD

cell provided by eNB or NG-RAN FDD cell provided by ng-eNB.";

reference "TS 28.658";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalEUtranCellFDDGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

grouping ExternalEUtranCellTDDWrapper {

list ExternalEUtranCellTDD {

description "Represents the common properties of external E-UTRAN cell

TDD provided by eNB or NG-RAN TDD cell provided by ng-eNB.";

reference "TS 28.658";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalEUtranCellTDDGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

augment "/subnet3gpp:SubNetwork/extenb3gpp:ExternalENBFunction" {

if-feature subnet3gpp:ExternalsUnderSubNetwork;

uses ExternalEUtranCellFDDWrapper;

}

augment "/eutranet3gpp:EUtraNetwork/extenb3gpp:ExternalENBFunction" {

if-feature eutranet3gpp:ExternalsUnderEUtraNetwork;

uses ExternalEUtranCellFDDWrapper;

}

augment "/subnet3gpp:SubNetwork/extenb3gpp:ExternalENBFunction" {

if-feature subnet3gpp:ExternalsUnderSubNetwork;

uses ExternalEUtranCellTDDWrapper;

}

augment "/eutranet3gpp:EUtraNetwork/extenb3gpp:ExternalENBFunction" {

if-feature eutranet3gpp:ExternalsUnderEUtraNetwork;

uses ExternalEUtranCellTDDWrapper;

}

}

## E.5.10	module _3gpp-nr-nrm-externalgnbcucpfunction@2019-10-28.yang

module _3gpp-nr-nrm-externalgnbcucpfunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-externalgnbcucpfunction";

prefix "extgnbcucp3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-nr-nrm-nrnetwork { prefix nrnet3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the ExternalGNBCUCPFunction

Information Object Class (IOC), that is part of the NR Network Resource

Model (NRM).";

reference "TS 28.541 5G Network Resource Model (NRM)";

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 {

description "Initial revision";

}

grouping ExternalGNBCUCPFunctionGrp {

description "Represets the ExternalGNBCUCPFunction IOC.";

reference "TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

leaf gNBId {

description "Identifies a gNB within a PLMN.";

reference "gNB Identifier (gNB ID) in TS 38.300, Global gNB ID

in TS 38.413";

mandatory true;

type int64 { range "0..4294967295"; }

}

leaf gNBIdLength {

description "Indicates the number of bits for encoding the gNB ID.";

reference "gNB ID in TS 38.300, Global gNB ID in TS 38.413";

mandatory true;

type int32 { range "22..32"; }

}

list pLMNId {

description "Specifies the PLMN identifier to be used as part of the

global RAN node identity.";

key "mcc mnc";

min-elements 1;

max-elements 1;

uses types3gpp:PLMNId;

}

}

grouping ExternalGNBCUCPFunctionWrapper {

list ExternalGNBCUCPFunction {

description "Represents the properties, known by the management function,

of a GNBCUCPFunction managed by another management function.";

reference "TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalGNBCUCPFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

augment "/subnet3gpp:SubNetwork" {

if-feature subnet3gpp:ExternalsUnderSubNetwork ;

uses ExternalGNBCUCPFunctionWrapper;

}

augment "/nrnet3gpp:NRNetwork" {

if-feature nrnet3gpp:ExternalsUnderNRNetwork;

uses ExternalGNBCUCPFunctionWrapper;

}

}

## E.5.11	module _3gpp-nr-nrm-externalgnbcuupfunction@2019-10-28.yang

module _3gpp-nr-nrm-externalgnbcuupfunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-externalgnbcuupfunction";

prefix "extgnbcuup3gpp";

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-nr-nrm-nrnetwork { prefix nrnet3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the ExternalGNBCUUPFunction

Information Object Class (IOC), that is part of the NR Network

Resource Model (NRM).";

reference "TS 28.541 5G Network Resource Model (NRM)";

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 {

description "Initial revision";

}

grouping ExternalGNBCUUPFunctionGrp {

description "Represets the ExternalGNBCUUPFunction IOC.";

reference "TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

leaf gNBId {

description "Identifies a gNB within a PLMN.";

reference "gNB Identifier (gNB ID) in TS 38.300, Global gNB ID

in TS 38.413";

mandatory true;

type int64 { range "0..4294967295"; }

}

leaf gNBIdLength {

description "Indicates the number of bits for encoding the gNB ID.";

reference "gNB ID in TS 38.300, Global gNB ID in TS 38.413";

mandatory true;

type int32 { range "22..32"; }

}

}

grouping ExternalGNBCUUPFunctionWrapper {

list ExternalGNBCUUPFunction {

description "Represents the properties, known by the management function,

of a GNBCUUPFunction managed by another management function.";

reference "TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalGNBCUUPFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

augment "/subnet3gpp:SubNetwork" {

if-feature subnet3gpp:ExternalsUnderSubNetwork ;

uses ExternalGNBCUUPFunctionWrapper;

}

augment "/nrnet3gpp:NRNetwork" {

if-feature nrnet3gpp:ExternalsUnderNRNetwork;

uses ExternalGNBCUUPFunctionWrapper;

}

}

## E.5.12	module _3gpp-nr-nrm-externalgnbdufunction@2019-10-28.yang

module _3gpp-nr-nrm-externalgnbdufunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-externalgnbdufunction";

prefix "extgnbdu3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-nr-nrm-nrnetwork { prefix nrnet3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the ExternalGNBDUFunction

Information Object Class (IOC) that is part of the NR Network Resource

Model (NRM).";

reference "TS 28.541 5G Network Resource Model (NRM)";

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 {

description "Initial revision";

}

grouping ExternalGNBDUFunctionGrp {

description "Represets the ExternalGNBDUFunction IOC.";

reference "TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

leaf gNBId {

description "Identifies a gNB within a PLMN.";

reference "gNB Identifier (gNB ID) in TS 38.300, Global gNB ID

in TS 38.413";

mandatory true;

type int64 { range "0..4294967295"; }

}

leaf gNBIdLength {

description "Indicates the number of bits for encoding the gNB ID.";

reference "gNB ID in TS 38.300, Global gNB ID in TS 38.413";

mandatory true;

type int32 { range "22..32"; }

}

list pLMNId {

description "Specifies the PLMN identifier to be used as part of the

global RAN node identity.";

key "mcc mnc";

min-elements 1;

max-elements 1;

uses types3gpp:PLMNId;

}

}

grouping ExternalGNBDUFunctionWrapper {

list ExternalGNBDUFunction {

description "Represents the properties, known by the management function,

of a GNBDUFunction managed by another management function.";

reference "TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalGNBDUFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

augment "/subnet3gpp:SubNetwork" {

if-feature subnet3gpp:ExternalsUnderSubNetwork ;

uses ExternalGNBDUFunctionWrapper;

}

augment "/nrnet3gpp:NRNetwork" {

if-feature nrnet3gpp:ExternalsUnderNRNetwork;

uses ExternalGNBDUFunctionWrapper;

}

}

## E.5.13	module _3gpp-nr-nrm-externalnrcellcu@2019-10-28.yang

module _3gpp-nr-nrm-externalnrcellcu {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-externalnrcellcu";

prefix "extnrcellcu3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-nr-nrm-nrnetwork { prefix nrnet3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-nr-nrm-externalgnbcucpfunction { prefix extgnbcucp3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the ExternalNRCellCU Information

Object Class (IOC), that is part of the NR Network Resource Model (NRM).";

reference "TS 28.541 5G Network Resource Model (NRM)";

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 {

description "Initial revision";

}

grouping ExternalNRCellCUGrp {

description "Represents the ExternalNRCellCU IOC.";

reference "TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

leaf cellLocalId {

description "Identifies an NR cell of a gNB. Together with corresponding

gNB ID it forms the NR Cell Identifier (NCI).";

reference "NCI in TS 38.300";

mandatory true;

type int32 {range "0..16383"; }

}

leaf nRPCI {

description "The Physical Cell Identity (PCI) of the NR cell.";

reference "TS 36.211";

mandatory true;

type int32 { range "0..1007"; }

}

list pLMNIdList {

description "Defines which PLMNs that are assumed to be served by the

NR cell in another gNB CU-CP. This list is either updated by the

managed element itself (e.g. due to ANR, signalling over Xn, etc.) or

by consumer over the standard interface.";

key "mcc mnc";

min-elements 1;

max-elements 12;

uses types3gpp:PLMNId;

}

leaf nRFrequencyRef {

description "Reference to corresponding NRFrequency instance.";

mandatory true;

type types3gpp:DistinguishedName;

}

}

grouping ExternalNRCellCUWrapper {

list ExternalNRCellCU {

description "Represents the properties of an NRCellCU controlled by

another Management Service Provider.";

reference "TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalNRCellCUGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

augment "/subnet3gpp:SubNetwork/extgnbcucp3gpp:ExternalGNBCUCPFunction" {

if-feature subnet3gpp:ExternalsUnderSubNetwork ;

uses ExternalNRCellCUWrapper;

}

augment "/nrnet3gpp:NRNetwork/extgnbcucp3gpp:ExternalGNBCUCPFunction" {

if-feature nrnet3gpp:ExternalsUnderNRNetwork;

uses ExternalNRCellCUWrapper;

}

}

## E.5.14	module _3gpp-nr-nrm-externalservinggwfunction@2019-10-28.yang

module _3gpp-nr-nrm-externalservinggwfunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-externalservinggwfunction";

prefix "extservgw3gpp";

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-nr-nrm-eutranetwork { prefix eutranet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the ExternalServingGWFunction

Information Object Class (IOC) that is part of the NR Network Resource

Model (NRM).";

reference "TS 28.541 5G Network Resource Model (NRM)";

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 {

description "Initial revision";

}

grouping ExternalServingGWFunctionGrp {

description "Represents the ExternalServingGWFunction IOC.";

reference "TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

}

grouping ExternalServingGWFunctionWrapper {

list ExternalServingGWFunction {

description "Represents the properties, known by the management

function, of a ServingGWFunction managed by another management

function.";

reference "TS 28.658";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalServingGWFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

augment "/subnet3gpp:SubNetwork" {

if-feature subnet3gpp:ExternalsUnderSubNetwork ;

uses ExternalServingGWFunctionWrapper;

}

augment "/eutranet3gpp:EUtraNetwork" {

if-feature eutranet3gpp:ExternalsUnderEUtraNetwork;

uses ExternalServingGWFunctionWrapper;

}

}

## E.5.15	module _3gpp-nr-nrm-externalupffunction@2019-10-28.yang

module _3gpp-nr-nrm-externalupffunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-externalupffunction";

prefix "extupf3gpp";

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-nr-nrm-nrnetwork { prefix nrnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the ExternalUPFFunction Information

Object Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "TS 28.541 5G Network Resource Model (NRM)";

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 {

description "Initial revision";

}

grouping ExternalUPFFunctionGrp {

description "Represents the ExternalUPFFunction IOC.";

reference "TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

}

grouping ExternalUPFFunctionWrapper {

list ExternalUPFFunction {

description "Represents the properties, known by the management

function, of a UPFFunction managed by another management

function.";

reference "TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalUPFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

augment "/subnet3gpp:SubNetwork" {

if-feature subnet3gpp:ExternalsUnderSubNetwork ;

uses ExternalUPFFunctionWrapper;

}

augment "/nrnet3gpp:NRNetwork" {

if-feature nrnet3gpp:ExternalsUnderNRNetwork;

uses ExternalUPFFunctionWrapper;

}

}

## E.5.16	module _3gpp-nr-nrm-gnbcucpfunction.yang

module _3gpp-nr-nrm-gnbcucpfunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-gnbcucpfunction";

prefix "gnbcucp3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-rrmpolicy { prefix nrrrmpolicy3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the GNBCUCPFunction Information

Object Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "TS 28.541 5G Network Resource Model (NRM)";

revision 2020-08-06 { reference "CR-0333"; }

revision 2020-08-03 { reference "CR-0321"; }

revision 2020-06-03 { reference "CR-0286"; }

revision 2020-05-08 { reference S5-203316 ; }

revision 2020-04-28 { reference "0260"; }

revision 2020-02-14 { reference S5-20XXXX ; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 {

description "Initial revision";

}

feature DESManagementFunction {

description "Classs representing Distributed SON or Domain-Centralized SON Energy Saving feature";

}

feature DANRManagementFunction {

description "Classs representing D-SON function of ANR Management feature";

}

feature DRACHOptimizationFunction {

description "Classs representing D-SON function of RACH optimization feature";

}

feature DMROFunction {

description "Classs representing D-SON function of MRO feature";

}

grouping GNBCUCPFunctionGrp {

description "Represents the GNBCUCPFunction IOC.";

reference "TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

uses nrrrmpolicy3gpp:RRMPolicy_Grp;

leaf gNBId {

description "Identifies a gNB within a PLMN. The gNB Identifier (gNB ID)

is part of the NR Cell Identifier (NCI) of the gNB cells.";

reference "gNB ID in TS 38.300, Global gNB ID in TS 38.413";

mandatory true;

type int64 { range "0..4294967295"; }

}

leaf gNBIdLength {

description "Indicates the number of bits for encoding the gNB ID.";

reference "gNB ID in TS 38.300, Global gNB ID in TS 38.413";

mandatory true;

type int32 { range "22..32"; }

}

leaf gNBCUName {

description "Identifies the Central Unit of an gNB.";

reference "TS 38.473";

mandatory true;

type string { length "1..150"; }

}

list pLMNId {

description "The PLMN identifier to be used as part of the global RAN

node identity.";

key "mcc mnc";

min-elements 1;

max-elements 1;

uses types3gpp:PLMNId;

}

leaf-list x2BlackList {

type string;

description "List of nodes to which X2 connections are prohibited.";

}

leaf-list x2WhiteList {

type string;

description "List of nodes to which X2 connections are enforced.";

}

leaf-list xnBlackList {

type string;

description "List of nodes to which Xn connections are prohibited.";

}

leaf-list xnWhiteList {

type string;

description "List of nodes to which X2 connections are enforced.";

}

leaf-list xnHOBlackList {

type string;

description "List of nodes to which handovers over  Xn are prohibited.";

}

leaf configurable5QISetRef {

type types3gpp:DistinguishedName;

description "DN of the Configurable5QISet that the GNBCUCPFunction supports (is associated to).";

}

leaf-list x2HOBlackList {

type string;

description "List of nodes to which handovers over X2 are prohibited.";

}

leaf dynamic5QISetRef {

type types3gpp:DistinguishedName;

description "DN of the Dynamic5QISet that the GNBCUCPFunction supports (is associated to).";

}

}

augment "/me3gpp:ManagedElement" {

list GNBCUCPFunction {

description "Represents the logical function CU-CP of gNB and en-gNB.";

reference "TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses GNBCUCPFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## E.5.17	module _3gpp-nr-nrm-gnbcuupfunction.yang

module _3gpp-nr-nrm-gnbcuupfunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-gnbcuupfunction";

prefix "gnbcuup3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-rrmpolicy { prefix nrrrmpolicy3gpp; }

import _3gpp-nr-nrm-common { prefix nrcommon3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the GNBCUUPFunction Information

Object Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "TS 28.541 5G Network Resource Model (NRM)";

revision 2020-08-06 { reference "CR-0333"; }

revision 2020-08-03 { reference "CR-0321"; }

revision 2020-06-03 { reference "CR-0286"; }

revision 2020-05-28 { reference "CR-0318"; }

revision 2020-03-12 { reference "SP-200233 S5-201547"; }

revision 2020-02-14 { reference S5-20XXXX ; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-08-21 {

description "Initial revision";

}

grouping TAIGrp {

description "Tracking Area Identity";

list pLMNId {

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf nRTAC {

type int64;

description "Identity of the common Tracking Area Code for the PLMNs

allowedValues:

a)	It is the TAC or Extended-TAC.

b)	A cell can only broadcast one TAC or Extended-TAC.

See TS 36.300, clause 10.1.7 (PLMNID and TAC relation).

c)	TAC is defined in clause 19.4.2.3 of TS 23.003 and

Extended-TAC is defined in clause 9.3.1.29 of TS 38.473.

d)	For a 5G SA (Stand Alone), it has a non-null value.";

}

}

grouping BackhaulAddressGrp {

description "Indicates the backhauladdress of gNB.";

leaf gNBId {

type uint32 {

range "0..4294967295";

}

description "It identifies a gNB within a PLMN. The gNB ID is part of

the NR Cell Identifier (NCI) of the gNB cells.";

reference "gNB Identifier (gNB ID) of clause 8.2 of TS 38.300.

Global gNB ID in clause 9.3.1.6 of TS 38.413";

}

list tAI {

key nRTAC;

min-elements 1;

max-elements 1;

description "Tracking Area Identity";

reference "clause 9.3.3.11 in TS 38.413";

uses TAIGrp;

}

}

grouping MappingSetIDBackhaulAddressGrp {

description "Mapping relationship between setID and backhaulAddress of gNB";

leaf idx {

type uint32 ;

description "ID value";

}

leaf setID {

type uint32;

mandatory true;

description "Indicates the setID of gNB.";

reference "Clause 7.4.1.6 in TS 38.211";

}

list backhaulAddress {

key gNBId;

min-elements 1;

max-elements 1;

description "Indicates the backhauladdress of gNB.";

uses BackhaulAddressGrp;

}

}

grouping GNBCUUPFunctionGrp {

description "Represents the GNBCUUPFunction IOC.";

reference "TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

uses nrrrmpolicy3gpp:RRMPolicy_Grp;

leaf gNBCUUPId {

type uint64 {

range "0..68719476735" ;

}

config false;

mandatory true;

description "Identifies the gNB-CU-UP at least within a gNB-CU-CP";

reference "'gNB-CU-UP ID' in clause 9.3.1.15 of TS 38.463";

}

leaf gNBId {

type uint32;

mandatory true;

description "Identifies a gNB within a PLMN. The gNB ID is part of the

NR Cell Identifier (NCI) of the gNB cells. ";

reference "gNB Identifier (gNB ID) of clause 8.2 of TS 38.300.

Global gNB ID in clause 9.3.1.6 of TS 38.413";

}

leaf gNBIdLength {

mandatory true;

type int32 { range "22..32"; }

description "Indicates the number of bits for encoding the gNB Id.";

reference "gNB Id in TS 38.300, Global gNB ID in TS 38.413";

}

list pLMNInfoList {

description "The PLMNInfoList is a list of PLMNInfo data type. It defines which PLMNs that

can be served by the GNBCUUPFunction and which S-NSSAIs can be supported by the

GNBCUUPFunction for corresponding PLMN in case of network slicing feature is supported";

key "mcc mnc";

uses nrcommon3gpp:PLMNInfo;

}

list mappingSetIDBackhaulAddressList {

key idx;

description "Specifies a list of mappingSetIDBackhaulAddress used to

retrieve the backhaul address of the victim set.

Must be present if Remote Interference Management function is supported.";

uses MappingSetIDBackhaulAddressGrp;

}

leaf configurable5QISetRef {

type types3gpp:DistinguishedName;

description "DN of the Configurable5QISet that the GNBCUUPFunction supports (is associated to).";

}

leaf dynamic5QISetRef {

type types3gpp:DistinguishedName;

description "DN of the Dynamic5QISet that the GNBCUUPFunction supports (is associated to).";

}

}

augment "/me3gpp:ManagedElement" {

list GNBCUUPFunction {

key id;

description "Represents the logical function CU-UP of gNB or en-gNB.";

reference "TS 28.541";

uses top3gpp:Top_Grp;

container attributes {

uses GNBCUUPFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## E.5.18	module_3gpp-nr-nrm-gnbdufunction@2020-02-14.yang

module _3gpp-nr-nrm-gnbdufunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-gnbdufunction";

prefix "gnbdu3gpp";

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-rrmpolicy { prefix nrrrmpolicy3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the GNBDUFunction Information

Object Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "TS 28.541 5G Network Resource Model (NRM)";

revision 2020-03-12 { reference "SP-200233 S5-201547" ; }

revision 2020-02-14 { reference S5-20XXXX ; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-08-21 {

description "Initial revision.";

}

grouping GNBDUFunctionGrp {

description "Represents the GNBDUFunction IOC.";

reference "TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

uses nrrrmpolicy3gpp:RRMPolicy_Grp;

leaf gNBId {

type int64 { range "0..4294967295"; }

config false;

mandatory true;

description "Identifies a gNB within a PLMN. The gNB Identifier (gNB ID)

is part of the NR Cell Identifier (NCI) of the gNB cells.";

reference "gNB ID in TS 38.300, Global gNB ID in TS 38.413";

}

leaf gNBIdLength {

type int32 { range "22..32"; }

mandatory true;

description "Indicates the number of bits for encoding the gNB ID.";

reference "gNB ID in TS 38.300, Global gNB ID in TS 38.413";

}

leaf gNBDUId {

type int64 { range "0..68719476735"; }

mandatory true;

description "Uniquely identifies the DU at least within a gNB.";

reference "TS 38.473";

}

leaf gNBDUName {

type string { length "1..150"; }

description "Identifies the Distributed Unit of an NR node";

reference "TS 38.473";

}

leaf aggressorSetID {

type uint32 { range "0..4194304"; }

config false;

description "Indicates the associated aggressor gNB Set ID of the cell

Valid when Remote Interference Management function is supported.";

reference "TS 38.211 clause 7.4.1.6";

}

leaf victimSetID {

type uint32 { range "0..4194304"; }

config false;

description "Indicates the associated victim gNB Set ID of the cell

Valid when Remote Interference Management function is supported.";

reference "TS 38.211 clause 7.4.1.6";

}

}

augment "/me3gpp:ManagedElement" {

list GNBDUFunction {

key id;

description "Represents the logical function DU of gNB or en-gNB.";

reference "TS 28.541";

uses top3gpp:Top_Grp;

container attributes {

uses GNBDUFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## E.5.19	module _3gpp-nr-nrm-nrcellcu@2020-02-14.yang

module _3gpp-nr-nrm-nrcellcu {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-nrcellcu";

prefix "nrcellcu3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-nr-nrm-gnbcucpfunction { prefix gnbcucp3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-common { prefix nrcommon3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the NRCellCU Information Object

Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "TS 28.541 5G Network Resource Model (NRM)";

revision 2020-05-08 { reference S5-203316 ; }

revision 2020-02-14 { reference S5-20XXXX ; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 {

description "Initial revision";

}

feature DESManagementFunction {

description "Classs representing Distributed SON or Domain-Centralized SON Energy Saving feature";

}

feature DRACHOptimizationFunction {

description "Classs representing D-SON function of RACH optimization feature";

}

feature DMROFunction {

description "Classs representing D-SON function of MRO feature";

}

feature CESManagementFunction {

description "Classs representing Cross Domain-Centralized SON Energy Saving feature";

}

grouping NRCellCUGrp {

description "Represents the NRCellCU IOC.";

reference "TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

leaf cellLocalId {

description "Identifies an NR cell of a gNB. Together with corresponding

gNB ID it forms the NR Cell Identifier (NCI).";

mandatory true;

type int32 { range "0..16383"; }

}

list pLMNInfoList {

description "The PLMNInfoList is a list of PLMNInfo data type. It defines which PLMNs

that can be served by the NR cell, and which S-NSSAIs that can be supported by the

NR cell for corresponding PLMN in case of network slicing feature is supported.";

// Note: Whether the attribute pLMNId in the pLMNInfo can be writable depends on the implementation.

key "mcc mnc";

min-elements 1;

uses nrcommon3gpp:PLMNInfo;

}

leaf nRFrequencyRef {

description "Reference to corresponding NRFrequency instance.";

config false;

type types3gpp:DistinguishedName;

}

}

augment "/me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction" {

list NRCellCU {

description "Represents the information required by CU that is

responsible for the management of inter-cell mobility and neighbour

relations via ANR.";

reference "TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NRCellCUGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## E.5.20	module _3gpp-nr-nrm-nrcelldu@2020-02-14.yang

module _3gpp-nr-nrm-nrcelldu {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-nrcelldu";

prefix "nrcelldu3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-gnbdufunction { prefix gnbdu3gpp; }

import _3gpp-nr-nrm-rrmpolicy { prefix nrrrmpolicy3gpp; }

import _3gpp-nr-nrm-common { prefix nrcommon3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the NRCellDU Information Object

Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "TS 28.541 5G Network Resource Model (NRM)";

revision 2020-05-08 { reference S5-203316 ; }

revision 2020-02-14 { reference S5-20XXXX ; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-09-03 {

description "Initial revision";

}

feature DPCIConfigurationFunction {

description "Classs representing Distributed SON or Domain-Centralized SON function of PCI configuration feature";

}

feature CPCIConfigurationFunction {

description "Classs representing Cross Domain-Centralized SON function of PCI configuration feature";

}

grouping NRCellDUGrp {

description "Represents the NRCellDU IOC.";

reference "TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

uses nrrrmpolicy3gpp:RRMPolicy_Grp;

leaf cellLocalId {

description "Identifies an NR cell of a gNB. Together with the

corresponding gNB identifier in forms the NR Cell Identity (NCI).";

reference "NCI in TS 38.300";

mandatory true;

type int32 { range "0..16383"; }

}

leaf operationalState  {

description "Operational state of the NRCellDU instance. Indicates

whether the resource is installed and partially or fully operable

(ENABLED) or the resource is not installed or not operable

(DISABLED).";

config false;

type types3gpp:OperationalState;

}

leaf administrativeState  {

description "Administrative state of the NRCellDU. Indicates the

permission to use or prohibition against using the cell, imposed

through the OAM services.";

type types3gpp:AdministrativeState;

default LOCKED;

}

leaf cellState  {

description "Cell state of the NRCellDU instance. Indicates whether the

cell is not currently in use (IDLE), or currently in use but not

configured to carry traffic (INACTIVE), or currently in use and is

configured to carry traffic (ACTIVE).";

config false;

type types3gpp:CellState;

}

list pLMNInfoList {

description "The PLMNInfoList is a list of PLMNInfo data type. It defines which PLMNs that

can be served by the NR cell, and which S-NSSAIs that can be supported by the NR cell for

corresponding PLMN in case of network slicing feature is supported. The plMNId of the first

entry of the list is the PLMNId used to construct the nCGI for the NR cell.";

key "mcc mnc";

min-elements 1;

uses nrcommon3gpp:PLMNInfo;

}

leaf nRPCI {

description "The Physical Cell Identity (PCI) of the NR cell.";

reference "TS 36.211";

mandatory true;

type int32 { range "0..1007"; }

}

leaf nRTAC {

description "The common 5GS Tracking Area Code for the PLMNs.";

reference "TS 23.003, TS 38.473";

type types3gpp:Tac;

}

leaf arfcnDL {

description "NR Absolute Radio Frequency Channel Number (NR-ARFCN) for

downlink.";

reference "TS 38.104";

mandatory true;

type int32;

}

leaf arfcnUL {

description "NR Absolute Radio Frequency Channel Number (NR-ARFCN) for

uplink.";

reference "TS 38.104";

type int32;

}

leaf arfcnSUL {

description "NR Absolute Radio Frequency Channel Number (NR-ARFCN) for

supplementary uplink.";

reference "TS 38.104";

type int32;

}

leaf bSChannelBwDL {

description "Base station channel bandwidth for downlink.";

reference "TS 38.104";

type int32;

units MHz;

}

leaf bSChannelBwUL {

description "Base station channel bandwidth for uplink.";

reference "TS 38.104";

type int32;

units MHz;

}

leaf bSChannelBwSUL {

description "Base station channel bandwidth for supplementary uplink.";

reference "TS 38.104";

mandatory false;

type int32;

units MHz;

}

leaf ssbFrequency {

description "Indicates cell defining SSB frequency domain position.

Frequency (in terms of NR-ARFCN) of the cell defining SSB transmission.

The frequency identifies the position of resource element RE=#0

(subcarrier #0) of resource block RB#10 of the SS block. The frequency

must be positioned on the NR global frequency raster, as defined in

TS 38.101-1, and within bSChannelBwDL.";

mandatory true;

type int32 { range "0..3279165"; }

}

leaf ssbPeriodicity {

description "Indicates cell defined SSB periodicity. The SSB periodicity

is used for the rate matching purpose.";

mandatory true;

type int32 { range "5 | 10 | 20 | 40 | 80 | 160"; }

units "subframes (ms)";

}

leaf ssbSubCarrierSpacing {

description "Subcarrier spacing of SSB. Only the values 15 kHz or 30 kHz

(< 6 GHz), 120 kHz or 240 kHz (> 6 GHz) are applicable.";

reference "TS 38.211";

mandatory true;

type int32 { range "15 | 30 | 120 | 240"; }

units kHz;

}

leaf ssbOffset {

description "Indicates cell defining SSB time domain position. Defined

as the offset of the measurement window, in which to receive SS/PBCH

blocks, where allowed values depend on the ssbPeriodicity

(ssbOffset < ssbPeriodicity).";

mandatory true;

type int32 { range "0..159"; }

units "subframes (ms)";

}

leaf ssbDuration {

description "Duration of the measurement window in which to receive

SS/PBCH blocks.";

reference "TS 38.213";

mandatory true;

type int32 { range "1..5"; }

units "subframes (ms)";

}

leaf-list nRSectorCarrierRef {

description "Reference to corresponding NRSectorCarrier instance.";

min-elements 1;

type types3gpp:DistinguishedName;

}

leaf-list bWPRef {

description "Reference to corresponding BWP instance.";

min-elements 0;

type types3gpp:DistinguishedName;

}

leaf-list nRFrequencyRef {

description "Reference to corresponding NRFrequency instance.";

min-elements 0;

type types3gpp:DistinguishedName;

}

}

augment "/me3gpp:ManagedElement/gnbdu3gpp:GNBDUFunction" {

list NRCellDU {

description "Represents the information of a cell known by DU.";

reference "TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NRCellDUGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## E.5.21	module _3gpp-nr-nrm-nrcellrelation@2019-10-28.yang

module _3gpp-nr-nrm-nrcellrelation {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-nrcellrelation";

prefix "nrcellrel3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-gnbcucpfunction { prefix gnbcucp3gpp; }

import _3gpp-nr-nrm-nrcellcu { prefix nrcellcu3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the NRCellRelation Information

Object Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "TS 28.541 5G Network Resource Model (NRM)";

revision 2020-06-03 { reference S5-202333 ; }

revision 2020-04-23 { reference CR0281 ; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-08-30 {

description "Initial revision";

}

typedef EnergySavingCoverage {

type enumeration {

enum FULL;

enum NO;

enum PARTIAL;

}

}

grouping NRCellRelationGrp {

description "Represents the NRCellRelation IOC.";

reference "TS 28.541";

leaf nRTCI {

description "Target NR Cell Identifier. It consists of NR Cell

Identifier (NCI) and Physical Cell Identifier of the target NR cell

(nRPCI).";

type uint64;

}

container cellIndividualOffset {

description "A set of offset values for the neighbour cell. Used when

UE is in connected mode. Defined for rsrpOffsetSSB, rsrqOffsetSSB,

sinrOffsetSSB, rsrpOffsetCSI-RS, rsrqOffsetCSI-RS and

sinrOffsetCSI-RS.";

reference "cellIndividualOffset in MeasObjectNR in TS 38.331";

leaf rsrpOffsetSsb {

description "Offset value of rsrpOffsetSSB.";

default 0;

type types3gpp:QOffsetRange;

}

leaf rsrqOffsetSsb{

description "Offset value of rsrqOffsetSSB.";

default 0;

type types3gpp:QOffsetRange;

}

leaf sinrOffsetSsb {

description "Offset value of sinrOffsetSSB.";

default 0;

type types3gpp:QOffsetRange;

}

leaf rsrpOffsetCsiRs{

description "Offset value of rsrpOffsetCSI-RS.";

default 0;

type types3gpp:QOffsetRange;

}

leaf rsrqOffsetCsiRs {

description "Offset value of rsrqOffsetCSI-RS.";

default 0;

type types3gpp:QOffsetRange;

}

leaf sinrOffsetCsiRs {

description "Offset value of sinrOffsetCSI-RS.";

default 0;

type types3gpp:QOffsetRange;

}

}

leaf nRFreqRelationRef {

description "Reference to a corresponding NRFreqRelation instance.";

mandatory true;

type types3gpp:DistinguishedName;

}

leaf adjacentNRCellRef {

description "Reference to an adjacent NR cell (NRCellCU or

ExternalNRCellCU).";

mandatory true;

type types3gpp:DistinguishedName;

}

leaf isRemoveAllowed {

type boolean;

default true;

description "True if the ANR function in the node is allowed to remove this relation.";

}

leaf isHOAllowed {

type boolean;

default true;

description "True if handovers are allowed over this relation.";

}

}

leaf isESCoveredBy {

description "Indicates whether the adjacent cell

provides no, partial or full coverage for the parent cell

instance. Adjacent cells with this attribute equal to FULL are

recommended to be considered as candidate cells to take over the

coverage when the original cell is about to be changed to energy

saving state. All adjacent cells with this property equal

to PARTIAL are recommended to be considered as entirety of candidate

cells to take over the coverage when the original cell is about to be

changed to energy saving state.";

type EnergySavingCoverage;

}

augment /me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction/nrcellcu3gpp:NRCellCU {

list NRCellRelation {

description "Represents a neighbour cell relation from a source cell

to a target cell, where the target cell is an NRCellCU or

ExternalNRCellCU instance.";

reference "TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NRCellRelationGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## E.5.22	module _3gpp-nr-nrm-nrfreqrelation@2019-10-28.yang

module _3gpp-nr-nrm-nrfreqrelation {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-nrfreqrelation";

prefix "nrfreqrel3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-gnbcucpfunction { prefix gnbcucp3gpp; }

import _3gpp-nr-nrm-nrcellcu { prefix nrcellcu3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the NRFreqRelation Information

Object Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "TS 28.541 5G Network Resource Model (NRM)";

revision 2020-04-23 { reference CR0281 ; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 {

description "Initial revision";

}

grouping NRFreqRelationGrp {

description "Represents the NRFreqRelation IOC.";

reference "TS 28.541";

container offsetMO {

description "A set of offset values applicable to all measured cells

with reference signal(s) indicated in corresponding MeasObjectNR. It

is used to indicate a cell, beam or measurement object specific offset

to be applied when evaluating candidates for cell re-selection or when

evaluating triggering conditions for measurement reporting. It is

defined for rsrpOffsetSSB, rsrqOffsetSSB, sinrOffsetSSB,

rsrpOffsetCSI-RS, rsrqOffsetCSI-RS and sinrOffsetCSI-RS.";

reference "offsetMO in MeasObjectNR in TS 38.331";

leaf rsrpOffsetSsb {

description "Offset value of rsrpOffsetSSB.";

default 0;

type types3gpp:QOffsetRange;

}

leaf rsrqOffsetSsb {

description "Offset value of rsrqOffsetSSB.";

default 0;

type types3gpp:QOffsetRange;

}

leaf sinrOffsetSsb {

description "Offset value of sinrOffsetSSB.";

default 0;

type types3gpp:QOffsetRange;

}

leaf rsrpOffsetCsiRs {

description "Offset value of rsrpOffsetCSI-RS.";

default 0;

type types3gpp:QOffsetRange;

}

leaf rsrqOffsetCsiRs {

description "Offset value of rsrqOffsetCSI-RS.";

default 0;

type types3gpp:QOffsetRange;

}

leaf sinrOffsetCsiRs {

description "Offset value of sinrOffsetCSI-RS.";

default 0;

type types3gpp:QOffsetRange;

}

}

leaf-list blackListEntry {

description "A list of Physical Cell Identities (PCIs) that are

blacklisted in NR measurements.";

reference "TS 38.331";

min-elements 0;

type uint16 { range "0..1007"; }

}

leaf-list blackListEntryIdleMode {

description "A list of Physical Cell Identities (PCIs) that are

blacklisted in SIB4 and SIB5.";

min-elements 0;

type uint16 { range "0..1007"; }

}

leaf cellReselectionPriority {

description "The absolute priority of the carrier frequency used by the

cell reselection procedure. Value 0 means lowest priority. The value

must not already used by other RAT, i.e. equal priorities between RATs

are not supported. The UE behaviour when no value is entered is

specified in clause 5.2.4.1 of TS 38.304.";

reference "CellReselectionPriority in TS 38.331, priority in

TS 38.304";

type uint32;

default 0;

}

leaf cellReselectionSubPriority {

description "Indicates a fractional value to be added to the value of

cellReselectionPriority to obtain the absolute priority of the

concerned carrier frequency for E-UTRA and NR.";

reference "TS 38.331";

type uint8 { range "2 | 4 | 6 | 8"; }

units "0.1";

}

leaf pMax {

description "Used for calculation of the parameter Pcompensation

(defined in TS 38.304), at cell reselection to a cell.";

reference "PEMAX in TS 38.101-1";

mandatory false;

type int32 { range "-30..33"; }

units dBm;

}

leaf qOffsetFreq {

description "The frequency specific offset applied when evaluating

candidates for cell reselection.";

mandatory false;

type types3gpp:QOffsetRange;

default 0;

}

leaf qQualMin {

description "Indicates the minimum required quality level in the cell.

Value 0 means that it is not sent and UE applies in such case the

(default) value of negative infinity for Qqualmin. Sent in SIB3 or

SIB5.";

reference "TS 38.304";

type int32 { range "-34..-3 | 0"; }

units dB;

default 0;

}

leaf qRxLevMin {

description "Indicates the required minimum received Reference Symbol

Received Power (RSRP) level in the NR frequency for cell reselection.

Broadcast in SIB3 or SIB5, depending on whether the related frequency

is intra- or inter-frequency. Resolution is 2.";

reference "TS 38.304";

mandatory true;

type int32 { range "-140..-44"; }

units dBm;

}

leaf threshXHighP {

description "Specifies the Srxlev threshold used by the UE when

reselecting towards a higher priority RAT/frequency than the current

serving frequency. Each frequency of NR and E-UTRAN might have a

specific threshold. Resolution is 2.";

reference "ThreshX, HighP in TS 38.304";

mandatory true;

type int32 { range "0..62"; }

units dB;

}

leaf threshXHighQ {

description "Specifies the Squal threshold used by the UE when

reselecting towards a higher priority RAT/frequency than the current

serving frequency. Each frequency of NR and E-UTRAN might have a

specific threshold.";

reference "ThreshX, HighQ in TS 38.304";

mandatory true;

type int32 { range "0..31"; }

units dB;

}

leaf threshXLowP {

description "Specifies the Srxlev threshold used by the UE when

reselecting towards a lower priority RAT/frequency than the current

serving frequency. Each frequency of NR and E-UTRAN might have a

specific threshold. Resolution is 2.";

reference "ThreshX, LowP in TS 38.304";

mandatory true;

type int32 { range "0..62"; }

units dB;

}

leaf threshXLowQ {

description "Specifies the Squal threshold used by the UE when

reselecting towards a lower priority RAT/frequency than the current

serving frequency. Each frequency of NR and E-UTRAN might have a

specific threshold.";

reference "ThreshX, LowQ in TS 38.304";

mandatory true;

type int32 { range "0..31"; }

units dB;

}

leaf tReselectionNR {

description "Cell reselection timer for NR.";

reference "TreselectionRAT for NR in TS 38.331";

mandatory true;

type int32 { range "0..7"; }

units s;

}

leaf tReselectionNRSfHigh {

description "The attribute tReselectionNr (parameter TreselectionNR in

TS 38.304) is multiplied with this scaling factor if the UE is

in high mobility state.";

reference "Speed dependent ScalingFactor for TreselectionNR for high

mobility state in TS 38.304";

mandatory true;

type uint8 { range "25 | 50 | 75 | 100"; }

units %;

}

leaf tReselectionNRSfMedium {

description "The attribute tReselectionNr (parameter TreselectionNR in

TS 38.304) multiplied with this scaling factor if the UE is in

medium mobility state.";

reference "Speed dependent ScalingFactor for TreselectionNR for medium

mobility state in TS 38.304";

mandatory true;

type uint8 { range "25 | 50 | 75 | 100"; }

units %;

}

leaf nRFrequencyRef {

description "Reference to a corresponding NRFrequency instance.";

mandatory true;

type types3gpp:DistinguishedName;

}

}

augment /me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction/nrcellcu3gpp:NRCellCU {

list NRFreqRelation {

description "Together with the target NRFrequency, it represents the

frequency properties applicable to the referencing NRFreqRelation.";

reference "TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NRFreqRelationGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## E.5.23	module _3gpp-nr-nrm-nrfrequency@2019-10-28.yang

module _3gpp-nr-nrm-nrfrequency {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-nrnetwork-nrfrequency";

prefix "nrfreq3gpp";

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-nr-nrm-nrnetwork { prefix nrnet3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the NRFrequency Information Object

Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "TS 28.541 5G Network Resource Model (NRM)";

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 {

description "Initial revision";

}

grouping NRFrequencyGrp {

description "Represents the NRFrequency IOC.";

reference "TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

leaf absoluteFrequencySSB {

description "The absolute frequency applicable for a downlink NR carrier

frequency associated with the SSB, in terms of NR-ARFCN.";

mandatory true;

type uint32 { range "0.. 3279165"; }

}

leaf sSBSubCarrierSpacing {

description "Sub-carrier spacing of the SSB.";

mandatory true;

type uint8 { range "15 | 30 | 60 | 120"; }

units "kHz";

}

leaf-list multiFrequencyBandListNR {

description "List of additional frequency bands the frequency belongs to.

The list is automatically set by the gNB.";

config false;

min-elements 0;

type uint16 { range "1..256"; }

}

}

grouping NRFrequencyWrapper {

list NRFrequency {

description "Represents certain NR frequency properties.";

reference "TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NRFrequencyGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

augment "/subnet3gpp:SubNetwork" {

if-feature subnet3gpp:ExternalsUnderSubNetwork ;

uses NRFrequencyWrapper;

}

augment "/nrnet3gpp:NRNetwork" {

if-feature nrnet3gpp:ExternalsUnderNRNetwork;

uses NRFrequencyWrapper;

}

}

## E.5.24	module _3gpp-nr-nrm-nrnetwork@2019-06-17.yang

module _3gpp-nr-nrm-nrnetwork {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-nrnetwork";

prefix "nrnet3gpp";

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the NRNetwork Information Object

Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "TS 28.541 5G Network Resource Model (NRM)";

revision 2019-06-17 {

description "Initial revision";

}

feature ExternalsUnderNRNetwork {

description "Classes representing external entities like NRFrequency,

ExternalGNBCUCPFunction, ExternalGNBDUFunction

are contained under a NRNetwork list/class.";

}

grouping NRNetworkGrp {

description "Represents the NRNetwork IOC.";

reference "TS 28.541";

uses subnet3gpp:SubNetworkGrp;

}

list NRNetwork {

description "A subnetwork containing gNB external NR entities.";

reference "TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NRNetworkGrp;

}

}

}

## E.5.25	module _3gpp-nr-nrm-nrsectorcarrier.yang

module _3gpp-nr-nrm-nrsectorcarrier {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-nrnetwork-nrsectorcarrier";

prefix "nrsectcarr3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-nr-nrm-gnbdufunction { prefix gnbdu3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the NRSectorCarrier Information

Object Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "TS 28.541 5G Network Resource Model (NRM)";

revision 2020-05-28 { reference CR-0316 ; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 {

description "Initial revision";

}

grouping NRSectorCarrierGrp {

description "Represents the NRSectorCarrier IOC.";

reference "TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

leaf txDirection {

description "Indicates if the transmission direction is downlink,

uplink, or both downlink and uplink.";

mandatory true;

type types3gpp:TxDirection;

}

leaf configuredMaxTxPower {

description "Maximum transmisssion power at the antenna port for all

downlink channels, used simultaneously in a cell, added together.

Condition:  The sector-carrier has a downlink and the

configuration of Tx power at antenna port reference point is supported.";

mandatory true;

type int32;

units mW;

}

leaf configuredMaxTxEIRP {

type int64;

units dBm;

mandatory true;

description "The maximum emitted isotroptic radiated power (EIRP) in dBm

for all downlink channels, used simultaneously in a cell, added together.

Condition: the sector-carrier has a downlink and the

configuration of emitted isotropic radiated power is supported";

}

leaf arfcnDL {

description "NR Absolute Radio Frequency Channel Number (NR-ARFCN)

for downlink.

Condition:  The sector-carrier has a downlink AND the value

differs from the referring cell's value of arfcnDL.";

reference "TS 38.104";

mandatory true;

type int32 { range "0..3279165"; }

}

leaf arfcnUL {

description "NR Absolute Radio Frequency Channel Number (NR-ARFCN)

for uplink.

Condition: The sector-carrier has an uplink AND the value

differs from the referring cell's value of arfcnUL.";

reference "TS 38.104";

mandatory true;

type int32 { range "0..3279165"; }

}

leaf bSChannelBwDL {

description "Base station channel bandwitdth for downlink.

Condition: The sector-carrier has a downlink AND the value

differs from the referring cell's value of bSChannelBwDL.";

reference "TS 38.104";

mandatory true;

type int32 { range "5 | 10 | 15 | 20 | 30 | 40 | 50 | 60 | 70 | 80 |

90 | 100"; }

units MHz;

}

leaf bSChannelBwUL {

description "Base station channel bandwitdth for uplink.";

reference "TS 38.104";

mandatory true;

type int32 { range "5 | 10 | 15 | 20 | 30 | 40 | 50 | 60 | 70 | 80 |

90 | 100"; }

units MHz;

}

leaf sectorEquipmentFunctionRef {

description "Reference to corresponding SectorEquipmentFunction

instance.";

reference "TS 23.622";

mandatory true;

type types3gpp:DistinguishedName;

}

}

augment "/me3gpp:ManagedElement/gnbdu3gpp:GNBDUFunction" {

list NRSectorCarrier {

description "Represents the resources of each transmission point

included in the cell.";

reference "TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NRSectorCarrierGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## E.5.26	module _3gpp-nr-nrm-rrmpolicy.yang

module _3gpp-nr-nrm-rrmpolicy {

yang-version 1.1;

namespace "urn:3gpp:sa5:3gpp-nr-nrm-nrnetwork-rrmpolicy";

prefix "nrrrmpolicy3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the RRMPolicy abstract class that is part of the NR Network Resource Model (NRM).";

reference "TS 28.541 5G Network Resource Model (NRM)";

revision 2020-04-28 {

reference "CR0285";

}

revision 2020-02-14 {

description "Initial revision";

}

grouping rRMPolicyMemberGrp {

description "This data type represents an RRM Policy member that will be part of a

rRMPolicyMemberList. A RRMPolicyMember is defined by its pLMNId and sNSSAI (S-NSSAI).

The members in a rRMPolicyMemberList are assigned a specific amount of RRM resources

based on settings in RRMPolicy.";

uses types3gpp:PLMNId;

leaf sNSSAI {

type types3gpp:SNssai;

}

}

typedef CyclicPrefix {

type enumeration {

enum NORMAL;

enum EXTENDED;

}

}

grouping RRMPolicy_Grp {

description "This IOC represents the properties of an abstract RRMPolicy. The RRMPolicy_ IOC

needs to be subclassed to be instantiated. It defines two attributes apart from those

inherited from Top IOC, the resourceType attribute defines type of resource (PRB, RRC

connected users, DRB usage etc.) and the rRMPolicyMemberList attribute defines the

RRMPolicyMember(s)that are subject to this policy. An RRM resource (defined in resourceType

attribute) is located in NRCellDU, NRCellCU, GNBDUFunction, GNBCUCPFunction or in

GNBCUUPFunction. The RRMPolicyRatio IOC is one realization of a RRMPolicy_ IOC, see the

inheritance in TS 28.541 Figure 4.2.1.2-1. This RRM framework allows adding new policies,

both standardized (like RRMPolicyRatio) or as vendor specific, by inheriting from the

abstract RRMPolicy_ IOC.";

leaf resourceType {

description "The resourceType attribute defines type of resource (PRB, RRC connected users,

DRB usage etc.) that is subject to policy. Valid values are 'PRB', 'RRC' or 'DRB'";

mandatory true;

type string;

}

list rRMPolicyMemberList{

description "It represents the list of RRMPolicyMember (s) that the managed object

is supporting. A RRMPolicyMember <<dataType>> include the PLMNId <<dataType>>

and S-NSSAI <<dataType>>." ;

min-elements 1;

key "idx";

leaf idx { type uint32; }

uses rRMPolicyMemberGrp;

}

} // grouping

grouping RRMPolicyRatioGrp {

description "Represents the RRMPolicyRatio concrete IOC.";

uses RRMPolicy_Grp;    // Inherits RRMPolicy_

leaf rRMPolicyMaxRatio {

description " This attribute specifies the maximum percentage of radio

resource that can be used by the associated rRMPolicyMemberList.

The maximum percentage of radio resource include at least one of

the shared resources, prioritized resources and dedicated resources.

The sum of the rRMPolicyMaxRatio values assigned to all RRMPolicyRatio(s)

name-contained by same ManagedEntity can be greater that 100.";

default 100;

type uint8 { range "0..100"; }

units percent;

}

leaf rRMPolicyMinRatio {

description " This attribute specifies the minimum percentage of radio resources

that can be used by the associated rRMPolicyMemberList. The minimum percentage

of radio resources including at least one of prioritized resources and dedicated

resources. The sum of the rRMPolicyMinRatio values assigned to all RRM PolicyRatio(s)

name-contained by same ManagedEntity shall be less or equal 100.";

default 0;

type uint8 { range "0..100"; }

units percent;

}

leaf rRMPolicyDedicatedRatio {

description " This attribute specifies the percentage of radio resource

that dedicatedly used by the associated rRMPolicyMemberList. The sum of

the rRMPolicyDeidctaedRatio values assigned to all RRMPolicyRatio(s)

name-contained by same ManagedEntity shall be less or equal 100. ";

default 0;

type uint8 { range "0..100"; }

units percent;

}

}

list RRMPolicyRatio {

description " The RRMPolicyRatio IOC is one realization of a RRMPolicy_ IOC, see the

inheritance in Figure 4.2.1.2-1. This RRM framework allows adding new policies, both

standardized (like RRMPolicyRatio) or as vendor specific, by inheriting from the

abstract RRMPolicy_ IOC. For details see clause 4.3.36.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses RRMPolicyRatioGrp;

}

}

}

## E.5.27	module _3gpp-nr-nrm-common@2020-02-14.yang

module _3gpp-nr-nrm-common {

yang-version 1.1;

namespace "urn:3gpp:sa5:3gpp-nr-nrm-common";

prefix "nrcommon3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of comon parts for TS 28.541.";

reference "TS 28.541 5G Network Resource Model (NRM)";

revision 2020-02-14 {

description "Initial revision";

}

grouping PLMNInfo {

description "The PLMNInfo data type define a S-NSSAI member in a specific PLMNId, and it have

two attributes PLMNId and S-NSSAI (PLMNId, S-NSSAI). The PLMNId represents a data type that

is comprised of mcc (mobile country code) and mnc (mobile network code), (See TS 23.003

clause 2.2 and 12.1) and S-NSSAI represents an data type, that is comprised of an SST

(Slice/Service type) and an optional SD (Slice Differentiator) field, (See TS 23.003 [13]).";

uses types3gpp:PLMNId;

leaf sNssai {

type types3gpp:SNssai;

}

}

}

## E.5.28	module _3gpp-nr-nrm-danrmanagementfunction.yang

module _3gpp-nr-nrm-danrmanagementfunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-danrmanagementfunction";

prefix "danrmanagementfunction3gpp";

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-gnbcucpfunction { prefix gnbcucp3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the DANRManagementFunction Information Object Class

(IOC) that is part of the NR Network Resource Model (NRM).";

reference "TS 28.541 5G Network Resource Model (NRM)";

revision 2020-05-08 { reference S5-203316; }

grouping DANRManagementFunctionGrp {

description "Represents the DANRManagementFunction IOC.";

reference "TS 28.541";

uses top3gpp:Top_Grp;

leaf intrasystemANRManagementSwitch {

description "This attribute determines whether the intra-system ANR function is activated or deactivated.";

type boolean;

}

leaf intersystemANRManagementSwitch {

description "This attribute determines whether the inter-system ANR function is activated or deactivated.";

type boolean;

}

}

augment "/me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction" {

if-feature gnbcucp3gpp:DANRManagementFunction;

uses DANRManagementFunctionGrp;

}

}

## E.5.29	module _3gpp-nr-nrm-desmanagementfunction.yang

module _3gpp-nr-nrm-desmanagementfunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-desmanagementfunction";

prefix "desmanagementfunction3gpp";

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-gnbcucpfunction { prefix gnbcucp3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-nr-nrm-nrcellcu { prefix nrcellcu3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the DESManagementFunction Information Object Class

(IOC) that is part of the NR Network Resource Model (NRM).";

reference "TS 28.541 5G Network Resource Model (NRM)";

revision 2020-05-08 { reference S5-203316; }

grouping DESManagementFunctionGrp {

description "Represents the DESManagementFunction IOC.";

reference "TS 28.541";

uses top3gpp:Top_Grp;

leaf desSwitch {

description "This attribute determines whether the Distributed SON or Domain-Centralized SON energy saving function is enabled or disabled.";

type boolean;

}

list intraRatEsActivationOriginalCellLoadParameters {

key loadThreshold;

description "This attributes is relevant, if the cell acts as an original cell.This attribute indicates the traffic load threshold and the time duration, which are used by distributed ES algorithms to allow a cell to enter the energySaving state.";

leaf loadThreshold {type int32;}

container attributes {

uses IntraRatEsActivationOriginalCellLoadParametersGrp;

}

}

list intraRatEsActivationCandidateCellsLoadParameters {

key loadThreshold;

description "This attribute indicates the traffic load threshold and the time duration, which are used by distributed ES algorithms level to allow a n 'original' cell to enter the energySaving state.";

leaf loadThreshold {type int32;}

container attributes {

uses IntraRatEsActivationCandidateCellsLoadParametersGrp;

}

}

list intraRatEsDeactivationCandidateCellsLoadParameters {

key loadThreshold;

description "This attributes is relevant, if the cell acts as a candidate cell.This attribute indicates the traffic load threshold  and the time duration which is used by distributed ES algorithms to allow a cell to leave the energySaving state.";

leaf loadThreshold {type int32;}

container attributes {

uses IntraRatEsDeactivationCandidateCellsLoadParametersGrp;

}

}

list esNotAllowedTimePeriod {

key startTimeandendTime;

description "This attribute indicates a list of time periods during which inter-RAT energy saving is not allowed.";

leaf startTimeandendTime {type string;}

container attributes {

uses EsNotAllowedTimePeriodGrp;

}

}

list interRatEsActivationOriginalCellParameters {

key loadThreshold;

description "This attribute indicates the traffic load threshold and the time duration, which are used by distributed inter-RAT ES algorithms to allow an original cell to enter the energySaving state.";

leaf loadThreshold {type int32;}

container attributes {

uses InterRatEsActivationOriginalCellParametersGrp;

}

}

list interRatEsActivationCandidateCellParameters {

key loadThreshold;

description "This attribute indicates the traffic load threshold and the time duration, which are used by distributed inter-RAT ES algorithms to allow an original cell to enter the energySaving state.";

leaf loadThreshold {type int32;}

container attributes {

uses InterRatEsActivationCandidateCellParametersGrp;

}

}

list interRatEsDeactivationCandidateCellParameters {

key loadThreshold;

description "This attribute indicates the traffic load threshold and the time duration which is used by distributed inter-RAT ES algorithms to allow an original cell to leave the energySaving state.";

leaf loadThreshold {type int32;}

container attributes {

uses InterRatEsDeactivationCandidateCellParametersGrp;

}

}

leaf energySavingState {

description "Specifies the status regarding the energy saving in the cell.";

type enumeration {

enum isNotEnergySaving;

enum isEnergySaving;

}

}

leaf isProbingCapable {

description " This attribute indicates whether this cell is capable of performing the ES probing procedure.";

type enumeration{

enum yes;

enum no;

}

}

}

grouping IntraRatEsActivationOriginalCellLoadParametersGrp {

description "Represents the the traffic load threshold and the time duration.";

leaf loadThreshold {

description "This attribute is used by distributed ES algorithms to allow a cell to enter the energySaving state.";

type int32 { range "0..10000"; }

units "1";

}

leaf timeDuration {

description " The time duration indicates how long the load needs to have been below the threshold.";

type int32 { range "0..900"; }

units "1";

}

}

grouping IntraRatEsActivationCandidateCellsLoadParametersGrp {

description "Represents the the traffic load threshold and the time duration.";

leaf loadThreshold {

description "This attribute is used by distributed ES algorithms to allow a cell to enter the energySaving state.";

type int32 { range "0..10000"; }

units "1";

}

leaf timeDuration {

description " The time duration indicates how long the load needs to have been below the threshold.";

type int32 { range "0..900"; }

units "1";

}

}

grouping IntraRatEsDeactivationCandidateCellsLoadParametersGrp {

description "Represents the the traffic load threshold and the time duration.";

leaf loadThreshold {

description "This attribute is used by distributed ES algorithms to allow a cell to enter the energySaving state.";

type int32 { range "0..10000"; }

units "1";

}

leaf timeDuration {

description " The time duration indicates how long the load needs to have been below the threshold.";

type int32 { range "0..900"; }

units "1";

}

}

grouping EsNotAllowedTimePeriodGrp {

description "Represents the the traffic load threshold and the time duration.";

leaf startTimeandendTime {

description "This field indicate valid UTC time.";

type string;

}

leaf periodOfDay {

description "This field indicate the period of day.";

type string;

}

leaf daysOfWeekList {

description "This field indicate the list of weekday.";

type string;

}

leaf listoftimeperiods {

description "This field indicate the list of time periods.";

type string;

}

}

grouping InterRatEsActivationOriginalCellParametersGrp {

description "Represents the the traffic load threshold and the time duration.";

leaf loadThreshold {

description "The time duration indicates how long the traffic load (both for UL and DL) needs to have been below the threshold.";

type int32 { range "0..10000"; }

units "1";

}

leaf timeDuration {

description " The time duration indicates how long the load needs to have been below the threshold.";

type int32 { range "0..900"; }

units "1";

}

}

grouping InterRatEsActivationCandidateCellParametersGrp {

description "Represents the the traffic load threshold and the time duration.";

leaf loadThreshold {

description "This attribute is used by distributed ES algorithms to allow a cell to enter the energySaving state.";

type int32 { range "0..10000"; }

units "1";

}

leaf timeDuration {

description "The time duration indicates how long the traffic load (both for UL and DL) in the candidate cell needs to have been below the threshold before any original cells which will be provided backup coverage by the candidate cell enters energySaving state.";

type int32 { range "0..900"; }

units "1";

}

}

grouping InterRatEsDeactivationCandidateCellParametersGrp {

description "Represents the the traffic load threshold and the time duration.";

leaf loadThreshold {

description "This attribute is used by distributed ES algorithms to allow a cell to enter the energySaving state.";

type int32 { range "0..10000"; }

units "1";

}

leaf timeDuration {

description "The time duration indicates how long the traffic load (either for UL or DL) in the candidate cell needs to have been above the threshold to wake up one or more original cells which have been provided backup coverage by the candidate cell.";

type int32 { range "0..900"; }

units "1";

}

}

augment "/me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction/nrcellcu3gpp:NRCellCU" {

if-feature nrcellcu3gpp:DESManagementFunction;

uses DESManagementFunctionGrp;

}

augment "/me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction" {

if-feature gnbcucp3gpp:DESManagementFunction;

uses DESManagementFunctionGrp;

}

augment "/me3gpp:ManagedElement" {

if-feature me3gpp:DESManagementFunction;

uses DESManagementFunctionGrp;

}

augment "/subnet3gpp:SubNetwork" {

if-feature subnet3gpp:DESManagementFunction;

uses DESManagementFunctionGrp;

}

}

## E.5.30	module _3gpp-nr-nrm-drachoptimizationfunction.yang

module _3gpp-nr-nrm-drachoptimizationfunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-drachoptimizationfunction";

prefix "drachoptimizationfunction3gpp";

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-gnbcucpfunction { prefix gnbcucp3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-nr-nrm-nrcellcu { prefix nrcellcu3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the DRACHOptimizationFunction Information Object Class

(IOC) that is part of the NR Network Resource Model (NRM).";

reference "TS 28.541 5G Network Resource Model (NRM)";

revision 2020-05-08 { reference S5-203316; }

grouping DRACHOptimizationFunctionGrp {

description "Represents the DRACHOptimizationFunction IOC.";

reference "TS 28.541";

uses top3gpp:Top_Grp;

list ueAccProbilityDistPerSSB {

key targetProbability;

description "This is a list of target Access Probability (APn) for the RACH optimization function.";

leaf targetProbability {type TargetProbability;}

container attributes {

uses UeAccProbilityDistPerSSBGrp;

}

}

list ueAccDelayProbilityDistPerSSB {

key targetProbability;

description "This is a list of target Access Delay probability (ADP) for the RACH optimization function.";

leaf targetProbability {type TargetProbability;}

container attributes {

uses UeAccDelayProbilityDistPerSSBGrp;

}

}

leaf drachOptimizationControl {

description "This attribute determines whether the RACH Optimization function is enabled or disabled.";

type boolean;

}

}

typedef TargetProbability {

type enumeration  {

enum 25;

enum 50;

enum 75;

enum 90;

}

}

typedef Numberofpreamblessent {

type int32  { range "1..200"; }

units "1";

}

typedef Accessdelay {

type int32  { range "10..560"; }

units "1";

}

grouping UeAccProbilityDistPerSSBGrp {

description "Represents the target Access Probability (APn) for the RACH optimization function.";

leaf targetProbability {

description "This attribute determines the target Probability.";

mandatory true;

type TargetProbability;

}

leaf numberofpreamblessent {

description "This attribute determines the number of preambles sent.";

mandatory true;

type Numberofpreamblessent;

}

}

grouping UeAccDelayProbilityDistPerSSBGrp {

description "Represents the target Access Delay probability (ADP) for the RACH optimization function.";

leaf targetProbability {

description "This attribute determines the target Probability.";

mandatory true;

type TargetProbability;

}

leaf accessdelay {

description "This attribute determines the access delay.";

mandatory true;

type Accessdelay;

}

}

augment "/me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction/nrcellcu3gpp:NRCellCU" {

if-feature nrcellcu3gpp:DRACHOptimizationFunction;

uses DRACHOptimizationFunctionGrp;

}

augment "/me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction" {

if-feature gnbcucp3gpp:DRACHOptimizationFunction;

uses DRACHOptimizationFunctionGrp;

}

augment "/me3gpp:ManagedElement" {

if-feature me3gpp:DRACHOptimizationFunction;

uses DRACHOptimizationFunctionGrp;

}

augment "/subnet3gpp:SubNetwork" {

if-feature nrcellcu3gpp:DRACHOptimizationFunction;

uses DRACHOptimizationFunctionGrp;

}

}

## E.5.31	module _3gpp-nr-nrm-dmrofunction.yang

module _3gpp-nr-nrm-dmrofunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-dmrofunction";

prefix "dmrofunction3gpp";

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-gnbcucpfunction { prefix gnbcucp3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-nr-nrm-nrcellcu { prefix nrcellcu3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the DMROFunction Information Object Class

(IOC) that is part of the NR Network Resource Model (NRM).";

reference "TS 28.541 5G Network Resource Model (NRM)";

revision 2020-05-08 { reference S5-203316; }

grouping DMROFunctionGrp {

description "Represents the DMROFunction IOC.";

reference "TS 28.541";

uses top3gpp:Top_Grp;

leaf maximumDeviationHoTrigger {

description "This parameter defines the maximum allowed absolute deviation of the Handover Trigger, from the default point of operation.";

type int32 { range "-20..20"; }

units "0.5";

}

leaf minimumTimeBetweenHoTriggerChange {

description "This parameter defines the minimum allowed time interval between two Handover Trigger change performed by MRO. This is used to control the stability and convergence of the algorithm.";

type int32 { range "0..604800"; }

units "1";

}

leaf tstoreUEcntxt {

description "The timer used for detection of too early HO, too late HO and HO to wrong cell.";

type int32 { range "0..1023"; }

units "100";

}

leaf dmroControl {

description " This attribute determines whether the MRO function is enabled or disabled.";

type boolean;

}

}

augment "/me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction/nrcellcu3gpp:NRCellCU" {

if-feature nrcellcu3gpp:DMROFunction;

uses DMROFunctionGrp;

}

augment "/me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction" {

if-feature gnbcucp3gpp:DMROFunction;

uses DMROFunctionGrp;

}

augment "/me3gpp:ManagedElement" {

if-feature me3gpp:DMROFunction;

uses DMROFunctionGrp;

}

augment "/subnet3gpp:SubNetwork" {

if-feature subnet3gpp:DMROFunction;

uses DMROFunctionGrp;

}

}

## E.5.32	module _3gpp-nr-nrm-dpciconfigurationfunction.yang

module _3gpp-nr-nrm-dpciconfigurationfunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-dpciconfigurationfunction";

prefix "dpciconfigurationfunction3gpp";

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-nrcelldu { prefix nrcelldu3gpp; }

import _3gpp-nr-nrm-gnbdufunction { prefix gnbdu3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the DPCIConfigurationFunction Information Object Class

(IOC) that is part of the NR Network Resource Model (NRM).";

reference "TS 28.541 5G Network Resource Model (NRM)";

revision 2020-05-08 { reference S5-203316; }

grouping DPCIConfigurationFunctionGrp {

description "Represents the DPCICONFIGURATIONFunction IOC.";

reference "TS 28.541";

uses top3gpp:Top_Grp;

list nRPciList {

key NRPci;

description "This holds a list of physical cell identities that can be assigned to the NR cells. This attribute shall be supported if D-SON PCI configuration or domain Centralized SON PCI configuration function is supported.";

leaf NRPci {type int32;}

container attributes {

uses NRPciListGrp;

}

}

leaf dPciConfigurationControl {

description " This attribute determines whether the Distributed SON or Domain-Centralized SON PCI configuration Function is enabled or disabled.";

type boolean;

}

}

grouping NRPciListGrp {

description "Represents the NR PCI list for the PCI configuration function.";

leaf NRPci {

description "This attribute determines the NR PCI.";

type int32 { range "0..1007"; }

units "1";

}

}

augment "/me3gpp:ManagedElement/gnbdu3gpp:GNBDUFunction/nrcelldu3gpp:NRCellDU" {

if-feature nrcelldu3gpp:DPCIConfigurationFunction;

uses DPCIConfigurationFunctionGrp;

}

augment "/me3gpp:ManagedElement" {

if-feature me3gpp:DPCIConfigurationFunction;

uses DPCIConfigurationFunctionGrp;

}

augment "/subnet3gpp:SubNetwork" {

if-feature subnet3gpp:DPCIConfigurationFunction;

uses DPCIConfigurationFunctionGrp;

}

}

## E.5.33	module _3gpp-nr-nrm-cpciconfigurationfunction.yang

module _3gpp-nr-nrm-cpciconfigurationfunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-cpciconfigurationfunction";

prefix "cpciconfigurationfunction3gpp";

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-nrcelldu { prefix nrcelldu3gpp; }

import _3gpp-nr-nrm-gnbdufunction { prefix gnbdu3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the CPCIConfigurationFunction Information Object Class

(IOC) that is part of the NR Network Resource Model (NRM).";

reference "TS 28.541 5G Network Resource Model (NRM)";

revision 2020-05-08 { reference S5-203316; }

grouping CPCIConfigurationFunctionGrp {

description "Represents the CPCICONFIGURATIONFunction IOC.";

reference "TS 28.541";

uses top3gpp:Top_Grp;

list cSonPciList {

key NRPci;

description " This holds a list of physical cell identities that can be assigned to the pci attribute by gNB. The assignment algorithm is not specified. This attribute shall be supported if and only if the C-SON PCI configuration is supported.";

leaf NRPci {type int32;}

container attributes {

uses CSonPciListGrp;

}

}

leaf cPciConfigurationControl {

description "This attribute determines whether the Cross Domain-Centralized SON PCI configuration function is enabled or disabled.";

type boolean;

}

}

grouping CSonPciListGrp {

description "Represents the C-SON PCI list for the PCI configuration function.";

leaf NRPci {

description "This attribute determines the NR PCI.";

type int32 { range "0..1007"; }

units "1";

}

}

augment "/me3gpp:ManagedElement/gnbdu3gpp:GNBDUFunction/nrcelldu3gpp:NRCellDU" {

if-feature nrcelldu3gpp:CPCIConfigurationFunction;

uses CPCIConfigurationFunctionGrp;

}

augment "/me3gpp:ManagedElement" {

if-feature me3gpp:CPCIConfigurationFunction;

uses CPCIConfigurationFunctionGrp;

}

augment "/subnet3gpp:SubNetwork" {

if-feature subnet3gpp:CPCIConfigurationFunction;

uses CPCIConfigurationFunctionGrp;

}

}

## E.5.34	module _3gpp-nr-nrm-cesmanagementfunction.yang

module _3gpp-nr-nrm-cesmanagementfunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-cesmanagementfunction";

prefix "cesmanagementfunction3gpp";

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-nrcellcu { prefix nrcellcu3gpp; }

import _3gpp-nr-nrm-gnbcucpfunction { prefix gnbcucp3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the CESManagementFunction Information Object Class

(IOC) that is part of the NR Network Resource Model (NRM).";

reference "TS 28.541 5G Network Resource Model (NRM)";

revision 2020-05-08 { reference S5-203316; }

grouping CESManagementFunctionGrp {

description "Represents the CESManagementFunction IOC.";

reference "TS 28.541";

uses top3gpp:Top_Grp;

leaf cesSwitch {

description "This attribute determines whether the Cross Domain-Centralized SON energy saving function is enabled or disabled.";

type boolean;

}

leaf energySavingState {

description "Specifies the status regarding the energy saving in the cell. If the value of energySavingControl is toBeEnergySaving, then it shall be tried to achieve the value isEnergySaving for the energySavingState. If the value of energySavingControl is toBeNotEnergySaving, then it shall be tried to achieve the value isNotEnergySaving for the energySavingState. ";

type enumeration{

enum isNotEnergySaving;

enum isEnergySaving;

}

}

leaf energySavingControl {

description "This attribute allows the Cross Domain-Centralized SON energy saving function to initiate energy saving activation or deactivation.";

type enumeration{

enum toBeEnergySaving;

enum toBeNotEnergySaving;

}

}

}

augment "/me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction/nrcellcu3gpp:NRCellCU" {

if-feature nrcellcu3gpp:CESManagementFunction;

uses CESManagementFunctionGrp;

}

augment "/me3gpp:ManagedElement" {

if-feature me3gpp:CESManagementFunction;

uses CESManagementFunctionGrp;

}

augment "/subnet3gpp:SubNetwork" {

if-feature subnet3gpp:CESManagementFunction;

uses CESManagementFunctionGrp;

}

}

# E.6	Void

# E.7	Mount information

At the mountpoint "children-of-SubNetwork" in the YANG module _3gpp-common-subnetwork, the following YANG modules may be mounted if the class ManagedElement and the underlying hierarchy is contained under a SubNetwork.

See [45] that describes the mechanism that adds the schema trees defined by a set of YANG modules onto a mount point defined in the schema tree in another YANG module.

_3gpp-common-ep-rp.yang

_3gpp-common-managed-element.yang

_3gpp-common-managed-function.yang

_3gpp-common-measurements.yang

_3gpp-common-subnetwork.yang

_3gpp-common-top.yang

_3gpp-common-yang-extensions.yang

_3gpp-common-yang-types.yang

_3gpp-nr-nrm-bwp.yang

_3gpp-nr-nrm-ep.yang

_3gpp-nr-nrm-eutrancellrelation.yang

_3gpp-nr-nrm-gnbcucpfunction.yang

_3gpp-nr-nrm-gnbcuupfunction.yang

_3gpp-nr-nrm-gnbdufunction.yang

_3gpp-nr-nrm-nrcellcu.yang

_3gpp-nr-nrm-nrcelldu.yang

_3gpp-nr-nrm-nrcellrelation.yang

_3gpp-nr-nrm-nrfreqrelation.yang

_3gpp-nr-nrm-nrfrequency.yang

_3gpp-nr-nrm-nrnetwork.yang

_3gpp-nr-nrm-nrsectorcarrier.yang

_3gpp-nr-nrm-beam.yang

_3gpp-nr-nrm-commonbeamformingfunction.yang

_3gpp-nr-nrm-rrmpolicy.yang

ietf-inet-types.yang

ietf-yang-types.yang

If the above files are mounted the yang files described in clause H.7 shall also be mounted.

Annex F (normative):
XML definitions for 5GC NRM

# F.1	General

This annex contains the XML definitions for the 5GC NRM specified in clause 5, in accordance with 5G NRM Information Model definitions specified in clause 4.

# F.2	Architectural features

The overall architectural feature of 5GC NRM information model is specified in clause 4, this clause specifies features that are specific to the Schema definitions.

The XML definitions of the present document specify the schema for a configuration content, which can be included in a configuration file for Bulk configuration management operations.

# F.3	Mapping

## F.3.1	General mapping

An IOC maps to an XML element of the same name as the IOC's name in the Information Model. An IOC attribute maps to a sub-element of the corresponding IOC's XML element, and the name of this sub-element is the same as the attribute's name in the Information Model.

## F.3.2	Information Object Class (IOC) mapping

The mapping is not present in the current version of the present document.

# F.4	Solution Set definitions

## F.4.1	XML definition structure

The overall description of the file format of configuration data XML files is provided by TS 32.616 [33].

The present document defines the NRM-specific XML schema ngcNrm.xsd for the 5GC NRM Information Model defined in clause 4.

XML schema ngcNrm.xsd explicitly declares NRM-specific XML element types for the related NRM.

The definition of those NRM-specific XML element types complies with the generic mapping rules defined in TS 32.616 [33].

## F.4.2	Graphical representation

The graphical representation is not present in the current version of the present document.

## F.4.3	XML schema "ngcNrm.xsd"

<?xml version="1.0" encoding="UTF-8"?>

<!--

TS 28.541 5GC Network Resource Model

XML schema definition

ngcNrm.xsd

-->

<schema

targetNamespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#ngcNrm"

elementFormDefault="qualified"

attributeFormDefault="unqualified"

xmlns="http://www.w3.org/2001/XMLSchema"

xmlns:xn="http://www.3gpp.org/ftp/specs/archive/28_series/28.623#genericNrm" xmlns:nn="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#nrNrm"     xmlns:en="http://www.3gpp.org/ftp/specs/archive/28_series/28.659#eutranNrm"

xmlns:ngc="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#ngcNrm"

>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.623#genericNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.659#eutranNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#nrNrm"/>

<!--NGC NRM IM class associated XML elements -->

<complexType name="aMFIdentifier">

<sequence>

<element name="amfRegionId" type="ngc:AmfRegionId"/>

<element name="amfSetId" type="ngc:AmfSetId"/>

<element name="amfPointer" type="ngc:AmfPointer"/>

</sequence>

</complexType>

<simpleType name="AmfRegionId">

<restriction base="integer">

<maxInclusive value="255"/>

<!-- The AMF Region ID is 8-bitslength, defined in 23.003 -->

</restriction>

</simpleType>

<simpleType name="AmfSetId">

<restriction base="integer">

<maxInclusive value="1023"/>

<!-- The AMF Region ID is 10-bits length, defined in 23.003 -->

</restriction>

</simpleType>

<simpleType name="AmfPointer">

<restriction base="integer">

<maxInclusive value="63"/>

<!-- The AMF Pointer is 6-bits length, defined in 23.003 -->

</restriction>

</simpleType>   <complexType name="NrTACList">

<sequence>

<element name="tac" type="nn:NrTac" minOccurs="0" maxOccurs="unbounded"/>

</sequence>

</complexType>

<complexType name="managedNFProfile">

<sequence>

<element name="nfInstanceID" type="string"/>

<element name="nfType" type="ngc:NfType"/>

<element name="hostAddr" type="ngc:hostAddr"/>

<element name="authzInfo" type="string" minOccurs="0"/>

<element name="location" type="string" minOccurs="0"/>

<element name="capacity" type="ngc:capacity" minOccurs="0"/>

<element name="nfInfo" type="ngc:Nfinfo"/>

</sequence>

</complexType>

<complexType name="hostAddr">

<!-- Refer to definitions in TS 28.541-->

<sequence>

<choice minOccurs="0" maxOccurs="1">

<element name="ipAddress" type="string"/>

<element name="fqdn" type="string"/>

</choice>

</sequence>

</complexType>

<simpleType name="capacity">

<!-- Refer to definitions in TS 28.541-->

<restriction base="integer">

<minInclusive value="0"/>

<maxInclusive value="65535"/>

</restriction>

</simpleType>

<complexType name="Nfinfo">

<!-- Refer to definitions in TS 28.541-->

<sequence>

<choice minOccurs="0" maxOccurs="1">

<element name="amfInfo" type="ngc:AmfInfo"/>

<element name="udrInfo" type="ngc:UdrInfo"/>

<element name="udmInfo" type="ngc:UdmInfo"/>

<element name="ausfInfo" type="ngc:AusfInfo"/>

<element name="upfInfo" type="ngc:UpfInfo"/>

</choice>

</sequence>

</complexType>

<complexType name="NFProfileList">

<sequence>

<element name="nfProfile" type="ngc:NfProfile"/>

</sequence>

</complexType>

<complexType name="NfProfile">

<sequence>

<element name="nfInstanceID" type="string"/>

<!-- nfInstanceID is uuid of NF instance -->

<element name="nfType" type="ngc:NfType"/>

<element name="sNssais" type="ngc: SnssaiList"/>

<element name="fqdn" type="string"/>

<element name="interPlmnFqdn" type="string"/>

<element name="ipv4Addresses" type="string"/>

<element name="ipv6Addresses" type="string"/>

<element name="ipv6Prefixes" type="string"/>

<element name="capacity" type="string"/>

<element name="udrInfo" type="ngc:UdrInfo"/>

<element name="amfInfo" type="ngc:AmfInfo"/>

<element name="smfInfo" type="ngc:SmfInfo"/>

<element name="upfInfo" type="ngc:UpfInfo"/>

<element name="nfServices" type="ngc:NfServices"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="nFSrvGroupId" type="string"/>

<element name="smfServingAreas" type="string"/>

<element name="locality" type="string"/>

<element name="authzInfo" type="string"/>

</sequence>

</complexType>

<complexType name="NfServices">

<sequence>

<element name="serviceInstanceId" type="string"/>

<element name="serviceName" type="string"/>

<element name="version" type="string"/>

<element name="schema" type="string"/>

<element name="fqdn" type="string"/>

<element name="interPlmnFqdn" type="string"/>

<element name="ipEndPoints" type="ngc:IpEndpoints"/>

<element name="apiPrefix" type="string"/>

<element name="defaultNotificationSubscriptions" type="ngc:DefaultNotificationSubscriptions"/>

<element name="allowedPlmns" type="nn:PLMNIdList"/>

<element name="allowedNfTypes" type="ngc:NFTypeList"/>

<element name="allowedNssais" type="ngc:Nssai"/>

<element name="capacity" type="string"/>

<element name="supportedFeatures" type="string"/>

</sequence>

</complexType>

<simpleType name="NfType">

<restriction base="string">

<!-- NF name is defined in TS 23.501 -->

<enumeration value="NRF"/>

<enumeration value="UDM"/>

<enumeration value="AMF"/>

<enumeration value="SMF"/>

<enumeration value="AUSF"/>

<enumeration value="NEF"/>

<enumeration value="PCF"/>

<enumeration value="SMSF"/>

<enumeration value="NSSF"/>

<enumeration value="UDR"/>

<enumeration value="LMF"/>

<enumeration value="GMLC"/>

<enumeration value="5GEIR"/>

<enumeration value="SEPP"/>

<enumeration value="UPF"/>

<enumeration value="N3IWF"/>

<enumeration value="AF"/>

<enumeration value="UDSF"/>

<enumeration value="DN"/>

</restriction>

</simpleType>

<complexType name="NFTypeList">

<sequence>

<element name="NFType" type="ngc:NfType"/>

</sequence>

</complexType>

<complexType name="LocalEndPoint">

<sequence>

<element name="ipv4Address" type="string"/>

<element name="ipv6Address" type="string"/>

<element name="ipv6Prefix" type="string"/>

<element name="vlanId" type="integer"/>

</sequence>

</complexType>

<complexType name="RemoteEndPoint">

<sequence>

<element name="ipv4Address" type="string"/>

<element name="ipv6Address" type="string"/>

<element name="ipv6Prefix" type="string"/>

</sequence>

</complexType>

<complexType name="UdrInfo">

<sequence>

<element name="supiRange" type="ngc:SupiRange"/>

</sequence>

</complexType>

<complexType name="SupiRange">

<sequence>

<element name="start" type="string"/>

<element name="end" type="string"/>

<element name="pattern" type="string"/>

</sequence>

</complexType>

<complexType name="AmfInfo">

<sequence>

<element name="amfSetId" type="ngc:AmfSetId"/>

</sequence>

</complexType>

<complexType name="SmfInfo">

<sequence>

<element name="dnn" type="string"/>

</sequence>

</complexType>

<complexType name="UpfInfo">

<sequence>

<element name="snssaiUpfInfo" type="ngc:SnssaiUpfInfo"/>

</sequence>

</complexType>

<complexType name="UdmInfo">

<sequence>

<element name="nFSrvGroupId" type="string"/>

</sequence>

</complexType>

<complexType name="AusfInfo">

<sequence>

<element name="nFSrvGroupId" type="string"/>

</sequence>

</complexType>

<complexType name="SnssaiUpfInfo">

<sequence>

<element name="sNssai" type="ngc:SNssai"/>

<element name="dnnUpfInfoList" type="ngc:DnnUpfInfoList"/>

</sequence>

</complexType>

<complexType name="DnnUpfInfoList">

<sequence>

<element name="dnn" type="string"/>

</sequence>

</complexType>

<complexType name="DefaultNotificationSubscription">

<sequence>

<element name="notificationType" type="ngc:NotificationType"/>

<element name="callbackUri" type="string"/>

<element name="n1MessageClass" type="string"/>

<element name="n2InformationClass" type="string"/>

</sequence>

</complexType>

<simpleType name="NotificationType">

<restriction base="string">

<enumeration value="N1_MESSAGES"/>

<enumeration value="N2_INFORMATION"/>

<enumeration value="LOCATION_NOTIFICATION"/>

</restriction>

</simpleType>

<simpleType name="TransportProtocol">

<restriction base="string">

<enumeration value="TCP"/>

</restriction>

</simpleType>

<simpleType name="NfStatus">

<restriction base="string">

<enumeration value="REGISTERED"/>

<enumeration value="SUSPENDED"/>

</restriction>

</simpleType>

<complexType name="NfRegistrationData">

<sequence>

<element name="heartBeatTimer" type="integer"/>

<element name="nfProfile" type="ngc:NfProfile"/>

</sequence>

</complexType>

<complexType name="CNSIIdList">

<sequence>

<element name="cNSIId" type="string"/>

<!-- CNSI Id is defined in TS 29.531 -->

</sequence>

</complexType>

<complexType name="SnssaiList">

<sequence>

<element name="sNssai" type="ngc:SNssai"/>

</sequence>

</complexType>

<complexType name="SNssai">

<sequence>

<element name="sst" type="ngc:Sst" minOccurs="0"/>

<element name="sd" type="ngc:Sd"/>

</sequence>

</complexType>

<simpleType name="Sst">

<restriction base="integer">

<maxInclusive value="255"/>

<!-- SST is 1-octets length and defined in TS 23.003 -->

</restriction>

</simpleType>

<simpleType name="Sd">

<restriction base="string">

<pattern value="^[A-Fa-f0-9]{6}$"/>

<!-- SST is 3-octets length and defined in TS 23.003 -->

</restriction>

</simpleType>

<simpleType name="WeightFactor">

<restriction base="integer">

</restriction>

</simpleType>

<simpleType name="SEPPType">

<restriction base="string">

<enumeration value="CSEPP"/>

<enumeration value="PSEPP"/>

</restriction>

</simpleType>

<complexType name="SupportedFunc">

<sequence>

<element name="function" type="string"/>

<element name="policy" type="string" minOccurs="0"/>

</sequence>

</complexType>

<complexType name="SupportedFuncList">

<sequence>

<element name="supportedFunc" type="ngc:SupportedFunc"/>

</sequence>

</complexType>

<simpleType name="CommModelType">

<restriction base="string">

<enumeration value="DIRECT_COMMUNICATION_WO_NRF"/>

<enumeration value="DIRECT_COMMUNICATION_WITH_NRF"/>

<enumeration value="INDIRECT_COMMUNICATION_WO_DEDICATED_DISCOVERY"/>

<enumeration value="INDIRECT_COMMUNICATION_WITH_DEDICATED_DISCOVERY"/>

</restriction>

</simpleType>

<complexType name="CommModel">

<sequence>

<element name="groupId" type="integer"/>

<element name="commModelType" type="ngc:CommModelType"/>

<element name="targetNFServiceList" type="xn:dnlist"/>

<element name="commModelConfiguration" type="string"/>

</sequence>

</complexType>

<complexType name="CommModelList">

<sequence>

<element name="commModel" type="ngc:CommModel"/>

</sequence>

</complexType>

<complexType name="CapabilityList">

<sequence>

<element name="capability" type="string"/>

</sequence>

</complexType>

<complexType name="FiveQIList">

<sequence>

<element name="FiveQI" type="integer"/>

</sequence>

</complexType>

<complexType name="FiveQiDscpMapping">

<sequence>

<element name="fiveQIValues" type="ngc:FiveQIList"/>

<element name="dscp" type="integer"/>

</sequence>

</complexType>

<complexType name="FiveQiDscpMappingList">

<sequence>

<element name="fiveQiDscpMapping" type="ngc:FiveDscpMapping"/>

</sequence>

</complexType>

<simpleType name="FiveQIResourceType">

<restriction base="string">

<enumeration value="GBR"/>

<enumeration value="NonGBR"/>

</restriction>

</simpleType>

<complexType name="PacketErrorRate">

<sequence>

<element name="scalar" type="integer"/>

<element name="exponent" type="integer"/>

</sequence>

</complexType>

<complexType name="FiveQICharacteristics">

<sequence>

<element name="fiveQIValue" type="integer"/>

<element name="resourceType" type="ngc:FiveQIResourceType"/>

<element name="priorityLevel" type="integer"/>

<element name="packetDelayBudget" type="integer"/>

<element name="packetErrorRate" type="ngc:PacketErrorRate "/>

<element name="averagingWindow" type="integer"/>

<element name="maximumDataBurstVolume" type="integer"/>

</sequence>

</complexType>

<complexType name="FiveQIList">

<sequence>

<element name="fiveQI" type="ngc:FiveQICharacteristics"/>

</sequence>

</complexType>

<simpleType name="GtpUPathQoSMonitoringStateType">

<restriction base="string">

<enumeration value="ENABLED"/>

<enumeration value="DISABLED"/>

</restriction>

</simpleType>

<complexType name="DscpList">

<sequence>

<element name="dscp" type="integer"/>

</sequence>

</complexType>

<complexType name="GtpUPathDelayThresholdsType">

<sequence>

<element name="n3AveragePacketDelayThreshold " type="integer"/>

<element name="n3MinPacketDelayThreshold" type="integer"/>

<element name="n3MaxPacketDelayThreshold" type="integer"/>

<element name="n9AveragePacketDelayThreshold " type="integer"/>

<element name="n9MinPacketDelayThreshold" type="integer"/>

<element name="n9MaxPacketDelayThreshold" type="integer"/>

</sequence>

</complexType>

<simpleType name="QFQoSMonitoringStateType">

<restriction base="string">

<enumeration value="ENABLED"/>

<enumeration value="DISABLED"/>

</restriction>

</simpleType>

<complexType name="FiveqiList">

<sequence>

<element name="FiveQI" type="integer"/>

</sequence>

</complexType>

<complexType name="QFPacketDelayThresholdsType">

<sequence>

<element name="thresholdDl" type="integer"/>

<element name="thresholUl" type="integer"/>

<element name="thresholdRtt" type="integer"/>

</sequence>

</complexType>

<simpleType name="AfSigProtocol">

<restriction base="string">

<enumeration value="NO_INFORMATION"/>

<enumeration value="SIP"/>

</restriction>

</simpleType>

<complexType name="PccRule">

<sequence>

<element name="pccRuleId" type="string"/>

<element name="flowInfoList" type="ngc:FlowInformationList"/>

<element name="applicationId" type="string"/>

<element name="appDescriptor" type="string" minOccurs="0"/>

<element name="contentVersion" type="integer" minOccurs="0"/>

<element name="precedence" type="integer"/>

<element name="afSigProtocol" type="ngc:AfSigProtocol" minOccurs="0"/>

<element name="isAppRelocatable" type="boolean" minOccurs="0"/>

<element name="isUeAddrPreserved" type="boolean" minOccurs="0"/>

<element name="qosData" type="ngc:QoSDataList"/>

<element name="altQosParams" type="ngc:QoSDataList" minOccurs="0"/>

<element name="trafficControlData" type="ngc:TrafficControlDataList"/>

<element name="conditionData" type="ngc:ConditionData" minOccurs="0"/>

<element name="tscaiInputUl" type="ngc:TscaiInputContainer" minOccurs="0"/>

<element name="tscaiInputDl" type="ngc:TscaiInputContainer" minOccurs="0"/>

</sequence>

</complexType>

<complexType name="PccRuleList">

<sequence>

<element name="pccRule" type="ngc:PccRule"/>

</sequence>

</complexType>

<simpleType name="FlowDirection">

<restriction base="string">

<enumeration value="DOWNLINK"/>

<enumeration value="UPLINK"/>

<enumeration value="BIDIRECTIONAL"/>

<enumeration value="UNSPECIFIED"/>

</restriction>

</simpleType>

<complexType name="FlowInformation">

<sequence>

<element name="flowDescription" type="string"/>

<element name="ethFlowDescription" type="ngc:EthFlowDescription"/>

<element name="packFiltId" type="string"/>

<element name="packetFilterUsage" type="boolean"/>

<element name="tosTrafficClass" type="string"/>

<element name="spi" type="string"/>

<element name="flowLabel" type="string" minOccurs="0"/>

<element name="flowDirection" type="ngc:FlowDirection"/>

</sequence>

</complexType>

<complexType name="FlowInformationList">

<sequence>

<element name="flowInfo" type="ngc:FlowInformation"/>

</sequence>

</complexType>

<simpleType name="FDir">

<restriction base="string">

<enumeration value="DOWNLINK"/>

<enumeration value="UPLINK"/>

</restriction>

</simpleType>

<complexType name="VlanTagList">

<sequence>

<element name="vlanTag" type="string"/>

</sequence>

</complexType>

<complexType name="EthFlowDescription">

<sequence>

<element name="destMacAddr" type="string"/>

<element name="ethType" type="string"/>

<element name="fDesc" type="string"/>

<element name="fDir" type="ngc:FDir"/>

<element name="sourceMacAddr" type="string"/>

<element name="vlanTags" type="ngc:VlanTagList"/>

<element name="srcMacAddrEnd" type="string" minOccurs="0"/>

<element name="destMacAddrEnd" type="string" minOccurs="0"/>

</sequence>

</complexType>

<complexType name="QoSData">

<sequence>

<element name="qosId" type="string"/>

<element name="fiveQIValue" type="integer"/>

<element name="maxbrUl" type="string" minOccurs="0"/>

<element name="maxbrDl" type="string" minOccurs="0"/>

<element name="gbrUl" type="string" minOccurs="0"/>

<element name="gbrDl" type="string" minOccurs="0"/>

<element name="arp" type="ngc:ARP"/>

<element name="qosNotificationControl" type="boolean" minOccurs="0"/>

<element name="reflectiveQos" type="boolean" minOccurs="0"/>

<element name="sharingKeyDl" type="string" minOccurs="0"/>

<element name="sharingKeyUl" type="string" minOccurs="0"/>

<element name="maxPacketLossRateDl" type="integer" minOccurs="0"/>

<element name="maxPacketLossRateUl" type="integer" minOccurs="0"/>

<element name="extMaxDataBurstVol" type="integer" minOccurs="0"/>

</sequence>

</complexType>

<complexType name="QoSDataList">

<sequence>

<element name="qoSData" type="ngc:QoSData"/>

</sequence>

</complexType>

<simpleType name="PreemptCap">

<restriction base="string">

<enumeration value="NOT_PREEMPT"/>

<enumeration value="MAY_PREEMPT"/>

</restriction>

</simpleType>

<simpleType name="PreemptVuln">

<restriction base="string">

<enumeration value="NOT_PREEMPTABLE"/>

<enumeration value="PREEMPTABLE"/>

</restriction>

</simpleType>

<complexType name="ARP">

<sequence>

<element name="priorityLevel" type="integer"/>

<element name="preemptCap" type="ngc:PreemptCap"/>

<element name="preemptVuln" type="ngc:PreemptVuln"/>

</sequence>

</complexType>

<simpleType name="FlowStatus">

<restriction base="string">

<enumeration value="ENABLED-UPLINK"/>

<enumeration value="ENABLED-DOWNLINK"/>

<enumeration value="ENABLED"/>

<enumeration value="DISABLED"/>

<enumeration value="REMOVED"/>

</restriction>

</simpleType>

<simpleType name="SteerFun">

<restriction base="string">

<enumeration value="MPTCP"/>

<enumeration value="ATSSS_LL"/>

</restriction>

</simpleType>

<complexType name="TrafficControlData">

<sequence>

<element name="tcId" type="string"/>

<element name="flowStatus" type="ngc:FlowStatus"/>

<element name="redirectInfo" type="ngc:RedirectInformation" minOccurs="0"/>

<element name="addRedirectInfo" type="ngc:RedirectInformationList" minOccurs="0"/>

<element name="muteNotif" type="boolean" minOccurs="0"/>

<element name="trafficSteeringPolIdDl" type="string" minOccurs="0"/>

<element name="trafficSteeringPolIdUl" type="string" minOccurs="0"/>

<element name="routeToLocs" type="ngc:RouteToLocationList"/>

<element name="upPathChgEvent" type="ngc:UpPathChgEvent" minOccurs="0"/>

<element name="steerFun" type="ngc:SteerFun" minOccurs="0"/>

<element name="steerModeDl" type="ngc:SteeringMode" minOccurs="0"/>

<element name="steerModeUl" type="ngc:SteeringMode" minOccurs="0"/>

<element name="mulAccCtrl" type="ngc:MulAccCtrl" minOccurs="0"/>

</sequence>

</complexType>

<complexType name="TrafficControlDataList">

<sequence>

<element name="trafficControlData" type="ngc:TrafficControlData"/>

</sequence>

</complexType>

<simpleType name="RedirectAddressType">

<restriction base="string">

<enumeration value="IPV4_ADDR"/>

<enumeration value="IPV6_ADDR"/>

<enumeration value="URL"/>

<enumeration value="SIP_URI"/>

</restriction>

</simpleType>

<complexType name="RedirectInformation">

<sequence>

<element name="redirectEnabled" type="boolean"/>

<element name="redirectAddressType" type="ngc:RedirectAddressType"/>

<element name="redirectServerAddress" type="string"/>

</sequence>

</complexType>

<complexType name="RedirectInformationList">

<sequence>

<element name="redirectInformation" type="ngc:RedirectInformation"/>

</sequence>

</complexType>

<complexType name="RouteToLocation">

<sequence>

<element name="dnai" type="string"/>

<element name="routeInfo" type="ngc:RouteInformation"/>

<element name="routeProfId" type="string"/>

</sequence>

</complexType>

<complexType name="RouteToLocationList">

<sequence>

<element name="routeToLocation" type="ngc:RouteToLocation"/>

</sequence>

</complexType>

<complexType name="RouteInformation">

<sequence>

<element name="ipv4Addr" type="string"/>

<element name="ipv6Addr" type="string"/>

<element name="portNumber" type="integer"/>

</sequence>

</complexType>

<simpleType name="DnaiChgType">

<restriction base="string">

<enumeration value="EARLY"/>

<enumeration value="EARLY_LATE"/>

<enumeration value="LATE"/>

</restriction>

</simpleType>

<complexType name="UpPathChgEvent">

<sequence>

<element name="notificationUri" type="string"/>

<element name="notifCorreId" type="string"/>

<element name="dnaiChgType" type="ngc:DnaiChgType"/>

<element name="afAckInd" type="boolean" minOccurs="0"/>

</sequence>

</complexType>

<simpleType name="SteerModeValue">

<restriction base="string">

<enumeration value="ACTIVE_STANDBY"/>

<enumeration value="LOAD_BALANCING"/>

<enumeration value="SMALLEST_DELAY"/>

<enumeration value="PRIORITY_BASED"/>

</restriction>

</simpleType>

<complexType name="SteeringMode">

<sequence>

<element name="steerModeValue" type="ngc:SteerModeValue"/>

<element name="active" type="ngc:AccessType"/>

<element name="standby" type="ngc:AccessType" minOccurs="0"/>

<element name="threeGLoad" type="integer"/>

<element name="prioAcc" type="ngc:AccessType"/>

</sequence>

</complexType>

<simpleType name="MulAccCtrl">

<restriction base="string">

<enumeration value="ALLOWED"/>

<enumeration value="NOT_ALLOWED"/>

</restriction>

</simpleType>

<simpleType name="RatType">

<restriction base="string">

<enumeration value="NR"/>

<enumeration value="EUTRA"/>

<enumeration value="WLAN"/>

<enumeration value="VIRTUAL"/>

<enumeration value="NBIOT"/>

<enumeration value="WIRELINE"/>

<enumeration value="WIRELINE_CABLE"/>

<enumeration value="WIRELINE_BBF"/>

<enumeration value="LTE-M"/>

<enumeration value="NR_U"/>

<enumeration value="EUTRA_U"/>

<enumeration value="TRUSTED_N3GA"/>

<enumeration value="TRUSTED_WLAN"/>

<enumeration value="UTRA"/>

<enumeration value="GERA"/>

</restriction>

</simpleType>

<simpleType name="AccessType">

<restriction base="string">

<enumeration value="3GPP_ACCESS"/>

<enumeration value="NON_3GPP_ACCESS"/>

</restriction>

</simpleType>

<complexType name="ConditionData">

<sequence>

<element name="condId" type="string"/>

<element name="activationTime" type="dateTime" minOccurs="0"/>

<element name="deactivationTime" type="dateTime" minOccurs="0"/>

<element name="accessType" type="ngc:AccessType" minOccurs="0"/>

<element name="ratType" type="ngc:RatType" minOccurs="0"/>

</sequence>

</complexType>

<complexType name="TscaiInputContainer">

<sequence>

<element name="periodicity" type="integer" minOccurs="0"/>

<element name="burstArrivalTime" type="dateTime" minOccurs="0"/>

</sequence>

</complexType>

<element name="AMFFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="nn:PLMNIdList"/>

<element name="aMFIdentifier" type="ngc:aMFIdentifier"/>

<element name="sBIFqdn" type="string"/>

<element name="snssaiList" type="ngc:SnssaiList" minOccurs="0"/>

<element name="aMFSet" type="xn:dn" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<element name="managedNFProfile" type="ngc:managedNFProfile" minOccurs="0"/>

<element name="commModelList" type="ngc:CommModelList" minOccurs="1"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N2"/>

<element ref="ngc:EP_N8"/>

<element ref="ngc:EP_N11"/>

<element ref="ngc:EP_N12"/>

<element ref="ngc:EP_N14"/>

<element ref="ngc:EP_N15"/>

<element ref="ngc:EP_N17"/>

<element ref="ngc:EP_N22"/>

<element ref="ngc:EP_N26"/>

<element ref="ngc:EP_N20"/>

<element ref="ngc:EP_NLS"/>

<element ref="ngc:EP_NLG"/>

<element ref="xn:VsDataContainer"/>

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="SMFFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="nRTACList" type="ngc:NrTACList"/>

<element name="sBIFqdn" type="string"/>

<element name="snssaiList" type="ngc:SnssaiList" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<element name="managedNFProfile" type="ngc:managedNFProfile" minOccurs="0"/>

<element name="commModelList" type="ngc:CommModelList" minOccurs="1"/>

<element name="configurable5QISetRef" type="xn:dn" minOccurs="0"/>

<element name="dynamic5QISetRef" type="xn:dn" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N4"/>

<element ref="ngc:EP_N10"/>

<element ref="ngc:EP_N11"/>

<element ref="ngc:EP_N7"/>

<element ref="ngc:EP_N16"/>

<element ref="ngc:EP_S5C"/>

<element ref="ngc:FiveQiDscpMappingSet"/>

<element ref="ngc:GtpUPathQoSMonitoringControl"/>

<element ref="ngc:QFQoSMonitoringControl"/>

<element ref="ngc:PredefinedPccRuleSet"/>              <element ref="xn:VsDataContainer"/>

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="UPFFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="nRTACList" type="ngc:NrTACList"/>

<element name="snssaiList" type="ngc:SnssaiList" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<element name="managedNFProfile" type="ngc:managedNFProfile" minOccurs="0"/>

<element name="commModelList" type="ngc:CommModelList" minOccurs="1"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N4"/>

<element ref="ngc:EP_N3"/>

<element ref="ngc:EP_N9"/>

<element ref="ngc:EP_S5U"/>

<element ref="ngc:EP_N6"/>

<element ref="xn:VsDataContainer"/>

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="N3IWFFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<element name="commModelList" type="ngc:CommModelList" minOccurs="1"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N2"/>

<element ref="ngc:EP_N3"/>

<element ref="xn:VsDataContainer"/>

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="PCFFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList" />

<element name="sBIFqdn" type="string" />

<element name="snssaiList" type="ngc:SnssaiList" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<element name="managedNFProfile" type="ngc:managedNFProfile" minOccurs="0"/>

<element name="commModelList" type="ngc:CommModelList" minOccurs="1"/>

<element name="configurable5QISetRef" type="xn:dn" minOccurs="0"/>

<element name="dynamic5QISetRef" type="xn:dn" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N7"/>

<element ref="ngc:EP_N15"/>

<element ref="ngc:EP_N16"/>

<element ref="ngc:EP_N5"/>

<element ref="ngc:EP_Rx"/>

<element ref="ngc:PredefinedPccRuleSet"/>

<element ref="xn:VsDataContainer"/>

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="AUSFFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="sBIFqdn" type="string"/>

<element name="snssaiList" type="ngc:SnssaiList" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<element name="managedNFProfile" type="ngc:managedNFProfile" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N12"/>

<element ref="ngc:EP_N13"/>

<element ref="xn:VsDataContainer"/>

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="UDMFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="sBIFqdn" type="string"/>

<element name="snssaiList" type="ngc:SnssaiList" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<element name="managedNFProfile" type="ngc:managedNFProfile" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N8"/>

<element ref="ngc:EP_N10"/>

<element ref="ngc:EP_N13"/>

<element ref="xn:VsDataContainer"/>

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="UDRFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="sBIFqdn" type="string"/>

<element name="snssaiList" type="ngc:SnssaiList" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<element name="managedNFProfile" type="ngc:managedNFProfile" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="UDSFFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="sBIFqdn" type="string"/>

<element name="snssaiList" type="ngc:SnssaiList" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<element name="managedNFProfile" type="ngc:managedNFProfile" minOccurs="0"/>

<element name="commModelList" type="ngc:CommModelList" minOccurs="1"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="NRFFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="sBIFqdn" type="string"/>

<element name="cNSIIdList" type="ngc:CNSIIdList" minOccurs="0"/>

<element name="nFProfileList" type="ngc:NFProfileList" minOccurs="0"/>

<element name="snssaiList" type="ngc:SnssaiList" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<element name="commModelList" type="ngc:CommModelList" minOccurs="1"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N27"/>

<element ref="xn:VsDataContainer"/>

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="NSSFFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="sBIFqdn" type="string"/>

<element name="cNSIIdList" type="ngc:CNSIIdList"/>

<element name="snssaiList" type="ngc: SnssaiList" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/> <element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<element name="managedNFProfile" type="ngc:managedNFProfile" minOccurs="0"/>

<element name="commModelList" type="ngc:CommModelList" minOccurs="1"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N27"/>

<element ref="ngc:EP_N31"/>

<element ref="xn:VsDataContainer"/>

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="SMSFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="sBIFqdn" type="string"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<element name="managedNFProfile" type="ngc:managedNFProfile" minOccurs="0"/>

<element name="commModelList" type="ngc:CommModelList" minOccurs="1"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N20"/>

<element ref="ngc:EP_N21"/>

<element ref="ngc:EP_MAP_SMSC"/>

<element ref="xn:VsDataContainer"/>

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="LMFFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<element name="managedNFProfile" type="ngc:managedNFProfile" minOccurs="0"/>

<element name="commModelList" type="ngc:CommModelList" minOccurs="1"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_NLS"/>

<element ref="xn:VsDataContainer"/>

<element ref="xn:MeasurementControl"/>            </choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="NGEIRFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="sBIFqdn" type="string"/>

<element name="snssaiList" type="ngc:SnssaiList" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<element name="managedNFProfile" type="ngc:managedNFProfile" minOccurs="0"/>               			<element name="commModelList" type="ngc:CommModelList" minOccurs="1"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N17"/>

<element ref="xn:VsDataContainer"/>

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="SEPPFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNId" type="en:PLMNId"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="sEPPType" type="nn:SEPPType"/>

<element name="sEPPId" type="integer"/>

<element name="fqdn" type="string"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N32"/>

<element ref="xn:VsDataContainer"/>

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="ExternalSEPPFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNId" type="en:PLMNId"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="sEPPId" type="integer"/>

<element name="fqdn" type="string"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="ngc:EP_N32"/>

<element ref="xn:VsDataContainer"/>

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="NWDAFFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="sBIFqdn" type="string"/>

<element name="snssaiList" type="ngc:SnssaiList" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<element name="managedNFProfile" type="ngc:managedNFProfile" minOccurs="0"/>

<element name="commModelList" type="ngc:CommModelList" minOccurs="1"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="SCPFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<element name="supportedFuncList" type="ngc:SupportedFuncList"/>

<element name="address" type="string"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="NEFFunction" substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="vnfParametersList" type="xn:vnfParametersListType" minOccurs="0"/>

<element name="priority" type="integer" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<element name="sBIFqdn" type="string"/>

<element name="snssaiList" type="ngc:SnssaiList" minOccurs="0"/>

<element name="managedNFProfile" type="ngc:ManagedNFProfile"/>

<element name="capabilitylist" type="ngc:CapabilityList"/>

<element name="isINEF" type="boolean"/>

<element name="isCAPIFSup" type="boolean"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N2">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="ngc:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="ngc:RemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N3">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="ngc:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="ngc:RemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N4">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="ngc:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="ngc:RemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N5">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="ngc:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="ngc:RemotePoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N6">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="ngc:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="ngc:RemotePoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N7">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="ngc:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="ngc:RemotePoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N8">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="ngc:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="ngc:RemotePoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N9">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="ngc:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="ngc:RemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N10">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="ngc:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="ngc:Remote" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N11">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="ngc:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="ngc:Remote" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N12">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="ngc:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="ngc:RemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N13">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="ngc:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="ngc:RemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N14">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="ngc:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="ngc:RemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N15">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="ngc:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="ngc:RemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N16">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="ngc:Local" minOccurs="0"/>

<element name="remoteAddress" type="ngc:RemotePoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N17">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="ngc:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="ngc:RemotePoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N20">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="ngc:Local" minOccurs="0"/>

<element name="remoteAddress" type="ngc:RemotePoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N21">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="ngc:Local" minOccurs="0"/>

<element name="remoteAddress" type="ngc:RemotePoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N22">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="ngc:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="ngc:RemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N26">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="ngc:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="ngc:RemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N27">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="ngc:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="ngc:RemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N31">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="ngc:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="ngc:RemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_N32">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="ngc:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="ngc:RemoteEndPoint" minOccurs="0"/>

<element name="remotePlmnId" type="en:PLMNId"/>

<element name="remoteSeppAddress" type="string"/>

<element name="remoteSeppId" type="integer" minOccurs="0"/>

<element name="n32cParas" type="string" minOccurs="0"/>

<element name="n32fPolicy" type="string" minOccurs="0"/>

<element name="withIPX" type="boolean"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_S5C">    <complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="ngc:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="ngc:RemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_S5U">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="ngc:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="ngc:RemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_Rx">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="ngc:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="ngc:RemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_MAP_SMSC">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="ngc:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="ngc:RemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_NLS">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="ngc:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="ngc:RemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_NLG">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<!-- Inherited attributes from EP_RP -->

<element name="farEndEntity" type="xn:dn" minOccurs="0"/>

<element name="userLabel" type="string" minOccurs="0"/>

<!-- End of inherited attributes from EP_RP -->

<element name="localAddress" type="ngc:LocalEndPoint" minOccurs="0"/>

<element name="remoteAddress" type="ngc:RemoteEndPoint" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="FiveQiDscpMappingSet">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="fiveQiDscpMappingList" type="ngc:FiveQiDscpMappingList"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="Configurable5QISet" substitutionGroup="xn:SubNetworkOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="configurable5QIs" type="ngc:FiveQIList"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="Dynamic5QISet" substitutionGroup="xn:SubNetworkOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="dynamic5QIs" type="ngc:FiveQIList"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="GtpUPathQoSMonitoringControl">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="gtpUPathQoSMonitoringState" type="ngc: GtpUPathQoSMonitoringStateType"/>

<element name="gtpUPathMonitoredSNSSAIs" type="ngc:SnssaiList"/>

<element name="monitoredDSCPs" type="ngc:DscpList"/>

<element name="isEventTriggeredGtpUPathMonitoringSupported" type="boolean"/>

<element name="isPeriodicGtpUMonitoringSupported" type="boolean"/>

<element name="isImmediateGtpUMonitoringSupported" type="boolean"/>

<element name="gtpUPathDelayThresholds" type="ngc:GtpUPathDelayThresholdsType" minOccurs="0"/>

<element name="gtpUPathMinimumWaitTime" type="integer" minOccurs="0"/>

<element name="gtpUPathMeasurementPeriod" type="integer" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="QFQoSMonitoringControl">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="qFQoSMonitoringState" type="ngc:QFQoSMonitoringStateType"/>

<element name="qFMonitoredSNSSAIs" type="ngc:SnssaiList"/>

<element name="qFMonitored5QIs" type="ngc:FiveqiList"/>

<element name="isEventTriggeredQFMonitoringSupported" type="boolean"/>

<element name="isPeriodicQFMonitoringSupported" type="boolean"/>

<element name="isSessionReleasedQFMonitoringSupported" type="boolean"/>

<element name="qFPacketDelayThresholds" type="ngc:QFPacketDelayThresholdsType" minOccurs="0"/>

<element name="qFMinimumWaitTime" type="integer" minOccurs="0"/>

<element name="qFMeasurementPeriod " type="integer" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="PredefinedPccRuleSet">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<element name="predefinedPccRules" type="ngc:PccRuleList"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

</schema>

Annex G (normative):
OpenAPI definition of the 5GC NRM

# G.1	General

This annex contains the OpenAPI definition of the NR NRM in YAML format.

The Information Service (IS) of the NR NRM is defined in clause 4.

Mapping rules to produce the OpenAPI definition based on the IS are defined in TS 32.160 [14].

# G.2	Void

# G.3	Void

# G.4	Solution Set (SS) definitions

## G.4.1	Void

## G.4.2	Void

## G.4.3	OpenAPI document "5gcNrm.yaml"

openapi: 3.0.1

info:

title: 3GPP 5GC NRM

version: 16.6.0

description: >-

OAS 3.0.1 specification of the 5GC NRM

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: TS 28.541 V16.6.0; 5G NRM, 5GC NRM

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.541/

paths: {}

components:

schemas:

#-------- Definition of types-----------------------------------------------------

AmfIdentifier:

type: object

description: 'AmfIdentifier comprise of amfRegionId, amfSetId and amfPointer'

properties:

amfRegionId:

$ref: '#/components/schemas/AmfRegionId'

amfSetId:

$ref: '#/components/schemas/AmfSetId'

amfPointer:

$ref: '#/components/schemas/AmfPointer'

AmfRegionId:

type: integer

description: AmfRegionId is defined in TS 23.003

maximum: 255

AmfSetId:

type: string

description: AmfSetId is defined in TS 23.003

maximum: 1023

AmfPointer:

type: integer

description: AmfPointer is defined in TS 23.003

maximum: 63

IpEndPoint:

type: object

properties:

ipv4Address:

$ref: 'genericNrm.yaml#/components/schemas/Ipv4Addr'

ipv6Address:

$ref: 'genericNrm.yaml#/components/schemas/Ipv6Addr'

ipv6Prefix:

$ref: 'genericNrm.yaml#/components/schemas/Ipv6Prefix'

transport:

$ref: 'genericNrm.yaml#/components/schemas/TransportProtocol'

port:

type: integer

NFProfileList:

type: array

description: List of NF profile

items:

$ref: '#/components/schemas/NFProfile'

NFProfile:

type: object

description: 'NF profile stored in NRF, defined in TS 29.510'

properties:

nFInstanceId:

type: string

description: uuid of NF instance

nFType:

$ref: 'genericNrm.yaml#/components/schemas/NFType'

nFStatus:

$ref: '#/components/schemas/NFStatus'

plmn:

$ref: 'nrNrm.yaml#/components/schemas/PlmnId'

sNssais:

$ref: 'nrNrm.yaml#/components/schemas/Snssai'

fqdn:

$ref: 'genericNrm.yaml#/components/schemas/Fqdn'

interPlmnFqdn:

$ref: 'genericNrm.yaml#/components/schemas/Fqdn'

nfServices:

type: array

items:

$ref: '#/components/schemas/NFService'

NFService:

type: object

description: NF Service is defined in TS 29.510

properties:

serviceInstanceId:

type: string

serviceName:

type: string

version:

type: string

schema:

type: string

fqdn:

$ref: 'genericNrm.yaml#/components/schemas/Fqdn'

interPlmnFqdn:

$ref: 'genericNrm.yaml#/components/schemas/Fqdn'

ipEndPoints:

type: array

items:

$ref: '#/components/schemas/IpEndPoint'

apiPrfix:

type: string

allowedPlmns:

$ref: 'nrNrm.yaml#/components/schemas/PlmnId'

allowedNfTypes:

type: array

items:

$ref: 'genericNrm.yaml#/components/schemas/NFType'

allowedNssais:

type: array

items:

$ref: 'nrNrm.yaml#/components/schemas/Snssai'

NFStatus:

type: string

description: any of enumrated value

enum:

- REGISTERED

- SUSPENDED

CNSIIdList:

type: array

items:

$ref: '#/components/schemas/CNSIId'

CNSIId:

type: string

description: CNSI Id is defined in TS 29.531, only for Core Network

TACList:

type: array

items:

$ref: 'nrNrm.yaml#/components/schemas/NrTac'

WeightFactor:

type: integer

UdmInfo:

type: object

properties:

nFSrvGroupId:

type: string

AusfInfo:

type: object

properties:

nFSrvGroupId:

type: string

UpfInfo:

type: object

properties:

smfServingAreas:

type: string

AmfInfo:

type: object

properties:

priority:

type: integer

SupportedDataSetId:

type: string

description: any of enumrated value

enum:

- SUBSCRIPTION

- POLICY

- EXPOSURE

- APPLICATION

Udrinfo:

type: object

properties:

supportedDataSetIds:

type: array

items:

$ref: '#/components/schemas/SupportedDataSetId'

nFSrvGroupId:

type: string

NFInfo:

oneOf:

- $ref: '#/components/schemas/UdmInfo'

- $ref: '#/components/schemas/AusfInfo'

- $ref: '#/components/schemas/UpfInfo'

- $ref: '#/components/schemas/AmfInfo'

- $ref: '#/components/schemas/Udrinfo'

ManagedNFProfile:

type: object

properties:

nfInstanceID:

type: string

nfType:

$ref: 'genericNrm.yaml#/components/schemas/NFType'

authzInfo:

type: string

hostAddr:

$ref: 'genericNrm.yaml#/components/schemas/HostAddr'

locality:

type: string

nFInfo:

$ref: '#/components/schemas/NFInfo'

capacity:

type: integer

SEPPType:

type: string

description: any of enumrated value

enum:

- CSEPP

- PSEPP

SupportedFunc:

type: object

properties:

function:

type: string

policy:

type: string

SupportedFuncList:

type: array

items:

$ref: '#/components/schemas/SupportedFunc'

CommModelType:

type: string

description: any of enumrated value

enum:

- DIRECT_COMMUNICATION_WO_NRF

- DIRECT_COMMUNICATION_WITH_NRF

- INDIRECT_COMMUNICATION_WO_DEDICATED_DISCOVERY

- INDIRECT_COMMUNICATION_WITH_DEDICATED_DISCOVERY

CommModel:

type: object

properties:

groupId:

type: integer

commModelType:

$ref: '#/components/schemas/CommModelType'

targetNFServiceList:

$ref: 'genericNrm.yaml#/components/schemas/DnList'

commModelConfiguration:

type: string

CommModelList:

type: array

items:

$ref: '#/components/schemas/CommModel'

CapabilityList:

type: array

items:

type: string

FiveQiDscpMapping:

type: object

properties:

fiveQIValues:

type: array

items:

type: integer

dscp:

type: integer

PacketErrorRate:

type: object

properties:

scalar:

type: integer

exponent:

type: integer

FiveQICharacteristics:

type: object

properties:

fiveQIValue:

type: integer

resourceType:

type: string

enum:

- GBR

- NonGBR

priorityLevel:

type: integer

packetDelayBudget:

type: integer

packetErrorRate:

$ref: '#/components/schemas/PacketErrorRate'

averagingWindow:

type: integer

maximumDataBurstVolume:

type: integer

GtpUPathDelayThresholdsType:

type: object

properties:

n3AveragePacketDelayThreshold:

type: integer

n3MinPacketDelayThreshold:

type: integer

n3MaxPacketDelayThreshold:

type: integer

n9AveragePacketDelayThreshold:

type: integer

n9MinPacketDelayThreshold:

type: integer

n9MaxPacketDelayThreshold:

type: integer

QFPacketDelayThresholdsType:

type: object

properties:

thresholdDl:

type: integer

thresholdUl:

type: integer

thresholdRtt:

type: integer

QosData:

type: object

properties:

qosId:

type: string

fiveQIValue:

type: integer

maxbrUl:

$ref: '/rep/all/5G_APIs/raw/master/TS29571_CommonData.yaml#/components/schemas/BitRateRm'

maxbrDl:

$ref: '/rep/all/5G_APIs/raw/master/TS29571_CommonData.yaml#/components/schemas/BitRateRm'

gbrUl:

$ref: '/rep/all/5G_APIs/raw/master/TS29571_CommonData.yaml#/components/schemas/BitRateRm'

gbrDl:

$ref: '/rep/all/5G_APIs/raw/master/TS29571_CommonData.yaml#/components/schemas/BitRateRm'

arp:

$ref: '/rep/all/5G_APIs/raw/master/TS29571_CommonData.yaml#/components/schemas/Arp'

qosNotificationControl:

type: boolean

reflectiveQos:

type: boolean

sharingKeyDl:

type: string

sharingKeyUl:

type: string

maxPacketLossRateDl:

$ref: '/rep/all/5G_APIs/raw/master/TS29571_CommonData.yaml#/components/schemas/PacketLossRateRm'

maxPacketLossRateUl:

$ref: '/rep/all/5G_APIs/raw/master/TS29571_CommonData.yaml#/components/schemas/PacketLossRateRm'

extMaxDataBurstVol:

$ref: '/rep/all/5G_APIs/raw/master/TS29571_CommonData.yaml#/components/schemas/ExtMaxDataBurstVolRm'

QosDataList:

type: array

items:

$ref: '#/components/schemas/QosData'

SteeringMode:

type: object

properties:

steerModeValue:

$ref: '/rep/all/5G_APIs/raw/master/TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/SteerModeValue'

active:

$ref: '/rep/all/5G_APIs/raw/master/TS29571_CommonData.yaml#/components/schemas/AccessType'

standby:

$ref: '/rep/all/5G_APIs/raw/master/TS29571_CommonData.yaml#/components/schemas/AccessTypeRm'

threeGLoad:

$ref: '/rep/all/5G_APIs/raw/master/TS29571_CommonData.yaml#/components/schemas/Uinteger'

prioAcc:

$ref: '/rep/all/5G_APIs/raw/master/TS29571_CommonData.yaml#/components/schemas/AccessType'

TrafficControlData:

type: object

properties:

tcId:

type: string

flowStatus:

$ref: '/rep/all/5G_APIs/raw/master/TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/FlowStatus'

redirectInfo:

$ref: '/rep/all/5G_APIs/raw/master/TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/RedirectInformation'

addRedirectInfo:

type: array

items:

$ref: '/rep/all/5G_APIs/raw/master/TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/RedirectInformation'

minItems: 1

muteNotif:

type: boolean

trafficSteeringPolIdDl:

type: string

nullable: true

trafficSteeringPolIdUl:

type: string

nullable: true

routeToLocs:

type: array

items:

$ref: '/rep/all/5G_APIs/raw/master/TS29571_CommonData.yaml#/components/schemas/RouteToLocation'

traffCorreInd:

type: boolean

upPathChgEvent:

$ref: '/rep/all/5G_APIs/raw/master/TS29571_CommonData.yaml#/components/schemas/UpPathChgEvent'

steerFun:

$ref: '/rep/all/5G_APIs/raw/master/TS29571_CommonData.yaml#/components/schemas/SteeringFunctionality'

steerModeDl:

$ref: '#/components/schemas/SteeringMode'

steerModeUl:

$ref: '#/components/schemas/SteeringMode'

mulAccCtrl:

$ref: '/rep/all/5G_APIs/raw/master/TS29571_CommonData.yaml#/components/schemas/MulticastAccessControl'

TrafficControlDataList:

type: array

items:

$ref: '#/components/schemas/TrafficControlData'

PccRule:

type: object

properties:

pccRuleId:

type: string

description: Univocally identifies the PCC rule within a PDU session.

flowInfoList:

type: array

items:

$ref: '/rep/all/5G_APIs/raw/master/TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/FlowInformation'

applicationId:

type: string

appDescriptor:

$ref: '/rep/all/5G_APIs/raw/master/TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/ApplicationDescriptor'

contentVersion:

$ref: '/rep/all/5G_APIs/raw/master/TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/ContentVersion'

precedence:

$ref: '/rep/all/5G_APIs/raw/master/TS29571_CommonData.yaml#/components/schemas/Uinteger'

afSigProtocol:

$ref: '/rep/all/5G_APIs/raw/master/TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/AfSigProtocol'

isAppRelocatable:

type: boolean

isUeAddrPreserved:

type: boolean

qosData:

type: array

items:

$ref: '#/components/schemas/QosDataList'

altQosParams:

type: array

items:

$ref: '#/components/schemas/QosDataList'

trafficControlData:

type: array

items:

$ref: '#/components/schemas/TrafficControlDataList'

conditionData:

$ref: '/rep/all/5G_APIs/raw/master/TS29512_Npcf_SMPolicyControl#/components/schemas/ConditionData'

tscaiInputDl:

$ref: '/rep/all/5G_APIs/raw/master/TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/TscaiInputContainer'

tscaiInputUl:

$ref: '/rep/all/5G_APIs/raw/master/TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/TscaiInputContainer'

#-------- Definition of concrete IOCs --------------------------------------------

SubNetwork-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/SubNetwork-Attr'

- $ref: 'genericNrm.yaml#/components/schemas/SubNetwork-ncO'

- type: object

properties:

SubNetwork:

$ref: '#/components/schemas/SubNetwork-Multiple'

ManagedElement:

$ref: '#/components/schemas/ManagedElement-Multiple'

ExternalAmfFunction:

$ref: '#/components/schemas/ExternalAmfFunction-Multiple'

ExternalNrfFunction:

$ref: '#/components/schemas/ExternalNrfFunction-Multiple'

ExternalNssfFunction:

$ref: '#/components/schemas/ExternalNssfFunction-Multiple'

AmfSet:

$ref: '#/components/schemas/AmfSet-Multiple'

AmfRegion:

$ref: '#/components/schemas/AmfRegion-Multiple'

Configurable5QISet:

$ref: '#/components/schemas/Configurable5QISet-Multiple'

Dynamic5QISet:

$ref: '#/components/schemas/Dynamic5QISet-Multiple'

ManagedElement-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedElement-Attr'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedElement-ncO'

- type: object

properties:

AmfFunction:

$ref: '#/components/schemas/AmfFunction-Multiple'

SmfFunction:

$ref: '#/components/schemas/SmfFunction-Multiple'

UpfFunction:

$ref: '#/components/schemas/UpfFunction-Multiple'

N3iwfFunction:

$ref: '#/components/schemas/N3iwfFunction-Multiple'

PcfFunction:

$ref: '#/components/schemas/PcfFunction-Multiple'

AusfFunction:

$ref: '#/components/schemas/AusfFunction-Multiple'

UdmFunction:

$ref: '#/components/schemas/UdmFunction-Multiple'

UdrFunction:

$ref: '#/components/schemas/UdrFunction-Multiple'

UdsfFunction:

$ref: '#/components/schemas/UdsfFunction-Multiple'

NrfFunction:

$ref: '#/components/schemas/NrfFunction-Multiple'

NssfFunction:

$ref: '#/components/schemas/NssfFunction-Multiple'

SmsfFunction:

$ref: '#/components/schemas/SmsfFunction-Multiple'

LmfFunction:

$ref: '#/components/schemas/LmfFunction-Multiple'

NgeirFunction:

$ref: '#/components/schemas/NgeirFunction-Multiple'

SeppFunction:

$ref: '#/components/schemas/SeppFunction-Multiple'

NwdafFunction:

$ref: '#/components/schemas/NwdafFunction-Multiple'

ScpFunction:

$ref: '#/components/schemas/ScpFunction-Multiple'

NefFunction:

$ref: '#/components/schemas/NefFunction-Multiple'

Configurable5QISet:

$ref: '#/components/schemas/Configurable5QISet-Multiple'

Dynamic5QISet:

$ref: '#/components/schemas/Dynamic5QISet-Multiple'

AmfFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

plmnIdList:

$ref: 'nrNrm.yaml#/components/schemas/PlmnIdList'

amfIdentifier:

$ref: '#/components/schemas/AmfIdentifier'

sBIFqdn:

type: string

weightFactor:

$ref: '#/components/schemas/WeightFactor'

snssaiList:

$ref: 'nrNrm.yaml#/components/schemas/SnssaiList'

amfSet:

$ref: 'genericNrm.yaml#/components/schemas/Dn'

managedNFProfile:

$ref: '#/components/schemas/ManagedNFProfile'

commModelList:

$ref: '#/components/schemas/CommModelList'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

EP_N2:

$ref: '#/components/schemas/EP_N2-Multiple'

EP_N8:

$ref: '#/components/schemas/EP_N8-Multiple'

EP_N11:

$ref: '#/components/schemas/EP_N11-Multiple'

EP_N12:

$ref: '#/components/schemas/EP_N12-Multiple'

EP_N14:

$ref: '#/components/schemas/EP_N14-Multiple'

EP_N15:

$ref: '#/components/schemas/EP_N15-Multiple'

EP_N17:

$ref: '#/components/schemas/EP_N17-Multiple'

EP_N20:

$ref: '#/components/schemas/EP_N20-Multiple'

EP_N22:

$ref: '#/components/schemas/EP_N22-Multiple'

EP_N26:

$ref: '#/components/schemas/EP_N26-Multiple'

EP_NLS:

$ref: '#/components/schemas/EP_NLS-Multiple'

EP_NLG:

$ref: '#/components/schemas/EP_NLG-Multiple'

AmfSet-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

plmnIdList:

$ref: 'nrNrm.yaml#/components/schemas/PlmnIdList'

nRTACList:

$ref: '#/components/schemas/TACList'

amfSetId:

$ref: '#/components/schemas/AmfSetId'

snssaiList:

$ref: 'nrNrm.yaml#/components/schemas/SnssaiList'

AmfRegion-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

plmnIdList:

$ref: 'nrNrm.yaml#/components/schemas/PlmnIdList'

nRTACList:

$ref: '#/components/schemas/TACList'

amfRegionId:

$ref: '#/components/schemas/AmfRegionId'

snssaiList:

$ref: 'nrNrm.yaml#/components/schemas/SnssaiList'

SmfFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

plmnIdList:

$ref: 'nrNrm.yaml#/components/schemas/PlmnIdList'

nRTACList:

$ref: '#/components/schemas/TACList'

sBIFqdn:

type: string

snssaiList:

$ref: 'nrNrm.yaml#/components/schemas/SnssaiList'

managedNFProfile:

$ref: '#/components/schemas/ManagedNFProfile'

commModelList:

$ref: '#/components/schemas/CommModelList'

configurable5QISetRef:

$ref: 'genericNRM.yaml#/components/schemas/Dn'

dynamic5QISetRef:

$ref: 'genericNRM.yaml#/components/schemas/Dn'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

EP_N4:

$ref: '#/components/schemas/EP_N4-Multiple'

EP_N7:

$ref: '#/components/schemas/EP_N7-Multiple'

EP_N10:

$ref: '#/components/schemas/EP_N10-Multiple'

EP_N11:

$ref: '#/components/schemas/EP_N11-Multiple'

EP_N16:

$ref: '#/components/schemas/EP_N16-Multiple'

EP_S5C:

$ref: '#/components/schemas/EP_S5C-Multiple'

FiveQiDscpMappingSet:

$ref: '#/components/schemas/FiveQiDscpMappingSet-Single'

GtpUPathQoSMonitoringControl:

$ref: '#/components/schemas/GtpUPathQoSMonitoringControl-Single'

QFQoSMonitoringControl:

$ref: '#/components/schemas/QFQoSMonitoringControl-Single'

PredefinedPccRuleSet:

$ref: '#/components/schemas/PredefinedPccRuleSet-Single'

UpfFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

plmnIdList:

$ref: 'nrNrm.yaml#/components/schemas/PlmnIdList'

nRTACList:

$ref: '#/components/schemas/TACList'

snssaiList:

$ref: 'nrNrm.yaml#/components/schemas/SnssaiList'

managedNFProfile:

$ref: '#/components/schemas/ManagedNFProfile'

commModelList:

$ref: '#/components/schemas/CommModelList'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

EP_N3:

$ref: '#/components/schemas/EP_N3-Multiple'

EP_N4:

$ref: '#/components/schemas/EP_N4-Multiple'

EP_N6:

$ref: '#/components/schemas/EP_N6-Multiple'

EP_N9:

$ref: '#/components/schemas/EP_N9-Multiple'

EP_S5U:

$ref: '#/components/schemas/EP_S5U-Multiple'

N3iwfFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

plmnIdList:

$ref: 'nrNrm.yaml#/components/schemas/PlmnIdList'

commModelList:

$ref: '#/components/schemas/CommModelList'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

EP_N3:

$ref: '#/components/schemas/EP_N3-Multiple'

EP_N4:

$ref: '#/components/schemas/EP_N4-Multiple'

PcfFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

plmnIdList:

$ref: 'nrNrm.yaml#/components/schemas/PlmnIdList'

sBIFqdn:

type: string

snssaiList:

$ref: 'nrNrm.yaml#/components/schemas/SnssaiList'

managedNFProfile:

$ref: '#/components/schemas/ManagedNFProfile'

commModelList:

$ref: '#/components/schemas/CommModelList'

configurable5QISetRef:

$ref: 'genericNRM.yaml#/components/schemas/Dn'

dynamic5QISetRef:

$ref: 'genericNRM.yaml#/components/schemas/Dn'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

EP_N5:

$ref: '#/components/schemas/EP_N5-Multiple'

EP_N7:

$ref: '#/components/schemas/EP_N7-Multiple'

EP_N15:

$ref: '#/components/schemas/EP_N15-Multiple'

EP_N16:

$ref: '#/components/schemas/EP_N16-Multiple'

EP_Rx:

$ref: '#/components/schemas/EP_Rx-Multiple'

PredefinedPccRuleSet:

$ref: '#/components/schemas/PredefinedPccRuleSet-Single'

AusfFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

plmnIdList:

$ref: 'nrNrm.yaml#/components/schemas/PlmnIdList'

sBIFqdn:

type: string

snssaiList:

$ref: 'nrNrm.yaml#/components/schemas/SnssaiList'

managedNFProfile:

$ref: '#/components/schemas/ManagedNFProfile'

commModelList:

$ref: '#/components/schemas/CommModelList'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

EP_N12:

$ref: '#/components/schemas/EP_N12-Multiple'

EP_N13:

$ref: '#/components/schemas/EP_N13-Multiple'

UdmFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

plmnIdList:

$ref: 'nrNrm.yaml#/components/schemas/PlmnIdList'

sBIFqdn:

type: string

snssaiList:

$ref: 'nrNrm.yaml#/components/schemas/SnssaiList'

managedNFProfile:

$ref: '#/components/schemas/ManagedNFProfile'

commModelList:

$ref: '#/components/schemas/CommModelList'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

EP_N8:

$ref: '#/components/schemas/EP_N8-Multiple'

EP_N10:

$ref: '#/components/schemas/EP_N10-Multiple'

EP_N13:

$ref: '#/components/schemas/EP_N13-Multiple'

UdrFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

plmnIdList:

$ref: 'nrNrm.yaml#/components/schemas/PlmnIdList'

sBIFqdn:

type: string

snssaiList:

$ref: 'nrNrm.yaml#/components/schemas/SnssaiList'

managedNFProfile:

$ref: '#/components/schemas/ManagedNFProfile'

UdsfFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

plmnIdList:

$ref: 'nrNrm.yaml#/components/schemas/PlmnIdList'

sBIFqdn:

type: string

snssaiList:

$ref: 'nrNrm.yaml#/components/schemas/SnssaiList'

managedNFProfile:

$ref: '#/components/schemas/ManagedNFProfile'

NrfFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

plmnIdList:

$ref: 'nrNrm.yaml#/components/schemas/PlmnIdList'

sBIFqdn:

type: string

cNSIIdList:

$ref: '#/components/schemas/CNSIIdList'

nFProfileList:

$ref: '#/components/schemas/NFProfileList'

snssaiList:

$ref: 'nrNrm.yaml#/components/schemas/SnssaiList'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

EP_N27:

$ref: '#/components/schemas/EP_N27-Multiple'

NssfFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

plmnIdList:

$ref: 'nrNrm.yaml#/components/schemas/PlmnIdList'

sBIFqdn:

type: string

cNSIIdList:

$ref: '#/components/schemas/CNSIIdList'

nFProfileList:

$ref: '#/components/schemas/NFProfileList'

snssaiList:

$ref: 'nrNrm.yaml#/components/schemas/SnssaiList'

commModelList:

$ref: '#/components/schemas/CommModelList'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

EP_N22:

$ref: '#/components/schemas/EP_N22-Multiple'

EP_N31:

$ref: '#/components/schemas/EP_N31-Multiple'

SmsfFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

plmnIdList:

$ref: 'nrNrm.yaml#/components/schemas/PlmnIdList'

sBIFqdn:

type: string

managedNFProfile:

$ref: '#/components/schemas/ManagedNFProfile'

commModelList:

$ref: '#/components/schemas/CommModelList'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

EP_N20:

$ref: '#/components/schemas/EP_N20-Multiple'

EP_N21:

$ref: '#/components/schemas/EP_N21-Multiple'

EP_MAP_SMSC:

$ref: '#/components/schemas/EP_MAP_SMSC-Multiple'

LmfFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

plmnIdList:

$ref: 'nrNrm.yaml#/components/schemas/PlmnIdList'

managedNFProfile:

$ref: '#/components/schemas/ManagedNFProfile'

commModelList:

$ref: '#/components/schemas/CommModelList'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

EP_NLS:

$ref: '#/components/schemas/EP_NLS-Multiple'

NgeirFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

plmnIdList:

$ref: 'nrNrm.yaml#/components/schemas/PlmnIdList'

sBIFqdn:

type: string

snssaiList:

$ref: 'nrNrm.yaml#/components/schemas/SnssaiList'

managedNFProfile:

$ref: '#/components/schemas/ManagedNFProfile'

commModelList:

$ref: '#/components/schemas/CommModelList'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

EP_N17:

$ref: '#/components/schemas/EP_N17-Multiple'

SeppFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

plmnId:

$ref: 'nrNrm.yaml#/components/schemas/PlmnId'

sEPPType:

$ref: '#/components/schemas/SEPPType'

sEPPId:

type: integer

fqdn:

$ref: 'genericNrm.yaml#/components/schemas/Fqdn'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

EP_N32:

$ref: '#/components/schemas/EP_N32-Multiple'

NwdafFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

plmnIdList:

$ref: 'nrNrm.yaml#/components/schemas/PlmnIdList'

sBIFqdn:

type: string

snssaiList:

$ref: 'nrNrm.yaml#/components/schemas/SnssaiList'

managedNFProfile:

$ref: '#/components/schemas/ManagedNFProfile'

commModelList:

$ref: '#/components/schemas/CommModelList'

ScpFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

supportedFuncList:

$ref: '#/components/schemas/SupportedFuncList'

address:

$ref: 'genericNrm.yaml#/components/schemas/HostAddr'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

NefFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

sBIFqdn:

type: string

snssaiList:

$ref: 'nrNrm.yaml#/components/schemas/SnssaiList'

managedNFProfile:

$ref: '#/components/schemas/ManagedNFProfile'

capabilityList:

$ref: '#/components/schemas/CapabilityList'

isINEF:

type: boolean

isCAPIFSup:

type: boolean

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

ExternalAmfFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

plmnIdList:

$ref: 'nrNrm.yaml#/components/schemas/PlmnIdList'

amfIdentifier:

$ref: '#/components/schemas/AmfIdentifier'

ExternalNrfFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

plmnIdList:

$ref: 'nrNrm.yaml#/components/schemas/PlmnIdList'

ExternalNssfFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

plmnIdList:

$ref: 'nrNrm.yaml#/components/schemas/PlmnIdList'

ExternalSeppFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

plmnId:

$ref: 'nrNrm.yaml#/components/schemas/PlmnId'

sEPPId:

type: integer

fqdn:

$ref: 'genericNrm.yaml#/components/schemas/Fqdn'

EP_N2-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_N3-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

epTransportRefs:

$ref: 'genericNrm.yaml#/components/schemas/DnList'

EP_N4-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_N5-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_N6-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_N7-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_N8-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_N9-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_N10-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_N11-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_N12-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_N13-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_N14-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_N15-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_N16-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_N17-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_N20-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_N21-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_N22-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_N26-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_N27-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_N31-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_N32-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

remotePlmnId:

$ref: 'nrNrm.yaml#/components/schemas/PlmnId'

remoteSeppAddress:

$ref: 'genericNrm.yaml#/components/schemas/HostAddr'

remoteSeppId:

type: integer

n32cParas:

type: string

n32fPolicy:

type: string

withIPX:

type: boolean

EP_S5C-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_S5U-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_Rx-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_MAP_SMSC-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_NLS-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_NLG-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

FiveQiDscpMappingSet-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

FiveQiDscpMappingList:

type: array

items:

$ref: '#/components/schemas/FiveQiDscpMapping'

Configurable5QISet-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

configurable5QIs:

type: array

items:

$ref: '#/components/schemas/FiveQICharacteristics'

Dynamic5QISet-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

dynamic5QIs:

type: array

items:

$ref: '#/components/schemas/FiveQICharacteristics'

GtpUPathQoSMonitoringControl-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

gtpUPathQoSMonitoringState:

type: string

enum:

- ENABLED

- DISABLED

gtpUPathMonitoredSNSSAIs:

type: array

items:

$ref: 'nrNrm.yaml#/components/schemas/Snssai'

monitoredDSCPs:

type: array

items:

type: integer

minimum: 0

maximum: 255

isEventTriggeredGtpUPathMonitoringSupported:

type: boolean

isPeriodicGtpUMonitoringSupported:

type: boolean

isImmediateGtpUMonitoringSupported:

type: boolean

gtpUPathDelayThresholds:

$ref: '#/components/schemas/GtpUPathDelayThresholdsType'

gtpUPathMinimumWaitTime:

type: integer

gtpUPathMeasurementPeriod:

type: integer

QFQoSMonitoringControl-Single:

allOf:

- $ref: 'genericNRM.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

qFQoSMonitoringState:

type: string

enum:

- ENABLED

- DISABLED

qFMonitoredSNSSAIs:

type: array

items:

$ref: 'nrNrm.yaml#/components/schemas/Snssai'

qFMonitored5QIs:

type: array

items:

type: integer

minimum: 0

maximum: 255

isEventTriggeredQFMonitoringSupported:

type: boolean

isPeriodicQFMonitoringSupported:

type: boolean

isSessionReleasedQFMonitoringSupported:

type: boolean

qFPacketDelayThresholds:

$ref: '#/components/schemas/QFPacketDelayThresholdsType'

qFMinimumWaitTime:

type: integer

qFMeasurementPeriod:

type: integer

PredefinedPccRuleSet-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

predefinedPccRules:

type: array

items:

$ref: '#/components/schemas/PccRule'

#-------- Definition of JSON arrays for name-contained IOCs ----------------------

SubNetwork-Multiple:

type: array

items:

$ref: '#/components/schemas/SubNetwork-Single'

ManagedElement-Multiple:

type: array

items:

$ref: '#/components/schemas/ManagedElement-Single'

AmfFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/AmfFunction-Single'

SmfFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/SmfFunction-Single'

UpfFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/UpfFunction-Single'

N3iwfFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/N3iwfFunction-Single'

PcfFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/PcfFunction-Single'

AusfFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/AusfFunction-Single'

UdmFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/UdmFunction-Single'

UdrFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/UdrFunction-Single'

UdsfFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/UdsfFunction-Single'

NrfFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/NrfFunction-Single'

NssfFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/NssfFunction-Single'

SmsfFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/SmsfFunction-Single'

LmfFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/LmfFunction-Single'

NgeirFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/NgeirFunction-Single'

SeppFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/SeppFunction-Single'

NwdafFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/NwdafFunction-Single'

ScpFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/ScpFunction-Single'

NefFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/NefFunction-Single'

ExternalAmfFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/ExternalAmfFunction-Single'

ExternalNrfFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/ExternalNrfFunction-Single'

ExternalNssfFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/ExternalNssfFunction-Single'

ExternalSeppFunction-Nultiple:

type: array

items:

$ref: '#/components/schemas/ExternalSeppFunction-Single'

AmfSet-Multiple:

type: array

items:

$ref: '#/components/schemas/AmfSet-Single'

AmfRegion-Multiple:

type: array

items:

$ref: '#/components/schemas/AmfRegion-Single'

EP_N2-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N2-Single'

EP_N3-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N3-Single'

EP_N4-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N4-Single'

EP_N5-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N5-Single'

EP_N6-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N6-Single'

EP_N7-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N7-Single'

EP_N8-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N8-Single'

EP_N9-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N9-Single'

EP_N10-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N10-Single'

EP_N11-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N11-Single'

EP_N12-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N12-Single'

EP_N13-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N13-Single'

EP_N14-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N14-Single'

EP_N15-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N15-Single'

EP_N16-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N16-Single'

EP_N17-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N17-Single'

EP_N20-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N20-Single'

EP_N21-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N21-Single'

EP_N22-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N22-Single'

EP_N26-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N26-Single'

EP_N27-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N27-Single'

EP_N31-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N31-Single'

EP_N32-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N32-Single'

EP_S5C-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_S5C-Single'

EP_S5U-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_S5U-Single'

EP_Rx-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_Rx-Single'

EP_MAP_SMSC-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_MAP_SMSC-Single'

EP_NLS-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_NLS-Single'

EP_NLG-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_NLG-Single'

Configurable5QISet-Multiple:

type: array

items:

$ref: '#/components/schemas/Configurable5QISet-Single'

Dynamic5QISet-Multiple:

type: array

items:

$ref: '#/components/schemas/Dynamic5QISet-Single'

#------------ Definitions in TS 28.541 for TS 28.532 -----------------------------

resources-5gcNrm:

oneOf:

- $ref: '#/components/schemas/SubNetwork-Single'

- $ref: '#/components/schemas/ManagedElement-Single'

- $ref: '#/components/schemas/AmfFunction-Single'

- $ref: '#/components/schemas/SmfFunction-Single'

- $ref: '#/components/schemas/UpfFunction-Single'

- $ref: '#/components/schemas/N3iwfFunction-Single'

- $ref: '#/components/schemas/PcfFunction-Single'

- $ref: '#/components/schemas/AusfFunction-Single'

- $ref: '#/components/schemas/UdmFunction-Single'

- $ref: '#/components/schemas/UdrFunction-Single'

- $ref: '#/components/schemas/UdsfFunction-Single'

- $ref: '#/components/schemas/NrfFunction-Single'

- $ref: '#/components/schemas/NssfFunction-Single'

- $ref: '#/components/schemas/SmsfFunction-Single'

- $ref: '#/components/schemas/LmfFunction-Single'

- $ref: '#/components/schemas/NgeirFunction-Single'

- $ref: '#/components/schemas/SeppFunction-Single'

- $ref: '#/components/schemas/NwdafFunction-Single'

- $ref: '#/components/schemas/ScpFunction-Single'

- $ref: '#/components/schemas/NefFunction-Single'

- $ref: '#/components/schemas/ExternalAmfFunction-Single'

- $ref: '#/components/schemas/ExternalNrfFunction-Single'

- $ref: '#/components/schemas/ExternalNssfFunction-Single'

- $ref: '#/components/schemas/ExternalSeppFunction-Single'

- $ref: '#/components/schemas/AmfSet-Single'

- $ref: '#/components/schemas/AmfRegion-Single'

- $ref: '#/components/schemas/QFQoSMonitoringControl-Single'

- $ref: '#/components/schemas/GtpUPathQoSMonitoringControl-Single'

- $ref: '#/components/schemas/EP_N2-Single'

- $ref: '#/components/schemas/EP_N3-Single'

- $ref: '#/components/schemas/EP_N4-Single'

- $ref: '#/components/schemas/EP_N5-Single'

- $ref: '#/components/schemas/EP_N6-Single'

- $ref: '#/components/schemas/EP_N7-Single'

- $ref: '#/components/schemas/EP_N8-Single'

- $ref: '#/components/schemas/EP_N9-Single'

- $ref: '#/components/schemas/EP_N10-Single'

- $ref: '#/components/schemas/EP_N11-Single'

- $ref: '#/components/schemas/EP_N12-Single'

- $ref: '#/components/schemas/EP_N13-Single'

- $ref: '#/components/schemas/EP_N14-Single'

- $ref: '#/components/schemas/EP_N15-Single'

- $ref: '#/components/schemas/EP_N16-Single'

- $ref: '#/components/schemas/EP_N17-Single'

- $ref: '#/components/schemas/EP_N20-Single'

- $ref: '#/components/schemas/EP_N21-Single'

- $ref: '#/components/schemas/EP_N22-Single'

- $ref: '#/components/schemas/EP_N26-Single'

- $ref: '#/components/schemas/EP_N27-Single'

- $ref: '#/components/schemas/EP_N31-Single'

- $ref: '#/components/schemas/EP_N31-Single'

- $ref: '#/components/schemas/EP_S5C-Single'

- $ref: '#/components/schemas/EP_S5U-Single'

- $ref: '#/components/schemas/EP_Rx-Single'

- $ref: '#/components/schemas/EP_MAP_SMSC-Single'

- $ref: '#/components/schemas/EP_NLS-Single'

- $ref: '#/components/schemas/EP_NLG-Single'

- $ref: '#/components/schemas/Configurable5QISet-Single'

- $ref: '#/components/schemas/FiveQiDscpMappingSet-Single'

- $ref: '#/components/schemas/PredefinedPccRuleSet-Single'

- $ref: '#/components/schemas/Dynamic5QISet-Single'

Annex H (normative):
YANG definitions for 5GC

# H.1	General

This annex contains the YANG definitions for the 5GC NRM, in accordance with 5GC information model definitions specified in clause 4.

# H.2	Void

# H.3	Void

# H.4	Void

# H.5	Modules

## H.5.1	module _3gpp-5gc-common-yang-types@2019-10-20.yang

module _3gpp-5g-common-yang-types {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-5g-common-yang-types";

prefix "types5g3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

organization "3GPP SA5";

description "The model defines common types for 5G networks and

network slicing.";

reference "TS 28.541";

revision 2019-10-20 {

description "Initial version.";

reference "Based on

TS 28.541 V16.X.XX";

}

typedef CommModelType {

reference "TS 23501";

type enumeration {

enum DIRECT_COMMUNICATION_WO_NRF {

value 0;

description "Directly communicate to other pre-configured NF service.";

}

enum DIRECT_COMMUNICATION_WITH_NRF {

value 1;

description "Directly communicate to other NF service discovered by NRF.";

}

enum INDIRECT_COMMUNICATION_WO_DEDICATED_DISCOVERY {

value 2;

description "Communicate to pre-configured other NF service through SCP as a proxy.";

}

enum INDIRECT_COMMUNICATION_WITH_DEDICATED_DISCOVERY {

value 3;

description "Communication to NF service discovered by NRF through SCP as a proxy.";

}

}

}

grouping CommModel {

leaf groupId {

type uint16;

}

leaf commModelType {

type CommModelType;

}

leaf-list targetNFServiceList {

type types3gpp:DistinguishedName;

}

leaf commModelConfiguration {

type string;

}

}

grouping SupportedFunc {

leaf function {

type string;

}

leaf policy {

type string;

}

}

}

## H.5.1a	module _3gpp-5gc-nrm-affunction@2019-10-28.yang

module _3gpp-5gc-nrm-affunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-affunction;

prefix af3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

description "This IOC is defined only to describe the IOCs representing

its interaction interface with 5GC (i.e. EP_Rx and EP_N5).

It has no attributes defined.";

reference "TS 28.541";

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-05-15 {

description "initial revision";

}

grouping AFFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

}

augment "/me3gpp:ManagedElement" {

list AFFunction {

description "5G Core AF Function";

reference "TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses AFFunctionGrp;

}

}

}

}

## H.5.2	module _3gpp-5gc-nrm-amffunction@2019-10-25.yang

module _3gpp-5gc-nrm-amffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-amffunction;

prefix amf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-top { prefix top3gpp; }

description "AMFFunction derived from basic ManagedFunction.";

revision 2019-10-25 { reference "S5-194457 S5-193518"; }

revision 2019-05-31 {

description "Ericsson refactoring.";

}

revision 2018-08-07 {

description "Initial revision";

}

grouping AMFFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

min-elements 1;

description "A list of PLMN identifiers (Mobile Country Code and Mobile Network Code).";

key "mcc mnc";

uses types3gpp:PLMNId;

}

container aMFIdentifier {

presence true;

description "An AMF identifier, comprising an AMF Region ID, an AMF Set ID and an AMF Pointer.";

uses types3gpp:AmfIdentifier;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based interface.";

type inet:domain-name;

}

leaf-list sNSSAIList {

min-elements 0;  // conditionally mandatory if network slicing feature is supported

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

reference "TS 23.003";

type types3gpp:SNssai;

}

list managedNFProfile {

key idx;

min-elements 1;

uses types3gpp:ManagedNFProfile;

}

list commModelList {

min-elements 1;

key "groupId";

uses types5g3gpp:CommModel;

}

}

augment "/me3gpp:ManagedElement" {

list AMFFunction {

description "5G Core AMF Function";

reference "TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses AMFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## H.5.3	module _3gpp-5gc-nrm-amfregion@2019-10-28.yang

module _3gpp-5gc-nrm-amfregion {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-amfregion;

prefix amfr3gpp;

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

description "This IOC represents the AMF Region which consists one or multiple AMF Sets.";

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-11 {

description "Ericsson refactoring.";

}

grouping AMFRegionGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf-list nRTACList {

description "List of Tracking Area Codes (legacy TAC or extended TAC)

where the represented management function is serving.";

reference "TS 38.413 clause 9.3.3.10";

min-elements 1;

config false;

type types3gpp:Tac;

}

leaf-list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//conditional support only if the network slicing feature is supported.

reference "TS 23.003";

type types3gpp:SNssai;

}

leaf aMFRegionId {

description "Represents the AMF Region ID, which identifies the region.";

mandatory true;

type types3gpp:AmfRegionId;

}

leaf-list aMFSet {

description "The AMFSet that the AFMRegion is associated with.";

min-elements 1;

type instance-identifier;

}

}

augment "/subnet3gpp:SubNetwork" {

list AMFRegion {

description "5G Core AMFRegion IOC";

reference "TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses AMFRegionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## H.5.4	module _3gpp-5gc-nrm-amfset@2019-10-28.yang

module _3gpp-5gc-nrm-amfset {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-amfset;

prefix amfset3gpp;

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

description "This IOC represents the AMF Set which consists of some AMFs that serve a given area and Network Slice.";

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-11 {

description "Ericsson refactoring.";

}

grouping AMFSetGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf-list nRTACList {

description "List of Tracking Area Codes (legacy TAC or extended TAC)

where the represented management function is serving.";

reference "TS 38.413 clause 9.3.3.10";

min-elements 1;

config false;

type types3gpp:Tac;

}

leaf-list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//conditional support only if the network slicing feature is supported.

reference "TS 23.003";

type types3gpp:SNssai;

}

leaf aMFRegion {

description "The AMFRegion that the AFMSet is associated with.";

type instance-identifier;

}

leaf-list aMFSetMemberList {

description "List of DNs of AMFFunction instances of the AMFSet.";

min-elements 1;

max-elements 1;

type types3gpp:DistinguishedName;

}

}

augment "/subnet3gpp:SubNetwork" {

list AMFSet {

description "5G Core AMFSet IOC";

reference "TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses AMFSetGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## H.5.5	module _3gpp-5gc-nrm-ausffunction@2019-10-25.yang

module _3gpp-5gc-nrm-ausffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-ausffunction;

prefix ausf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

description "This IOC represents the AUSF function in 5GC. For more information about the AUSF, see TS 23.501.";

reference "TS 28.541";

revision 2019-10-25 { reference "S5-194457 S5-193518"; }

revision 2019-05-22 {

description "initial revision";

}

grouping AUSFFuntionGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based interface.";

type inet:domain-name;

}

leaf-list  sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//optional support

reference "TS 23.003";

type types3gpp:SNssai;

}

list managedNFProfile {

key idx;

min-elements 1;

uses types3gpp:ManagedNFProfile;

}

list commModelList {

min-elements 1;

key "groupId";

uses types5g3gpp:CommModel;

}

}

augment "/me3gpp:ManagedElement" {

list AUSFFunction {

description "5G Core AUSF Function";

reference "TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses AUSFFuntionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## H.5.6	module _3gpp-5gc-nrm-dnfunction@2019-10-28.yang

module _3gpp-5gc-nrm-dnfunction {

yang-version 1.1;

namespace urn:3gpp:sa5_3gpp-5gc-nrm-dnfunction;

prefix dn3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

description "This IOC is defined only to describe the IOCs representing

Data Network (DN) interaction interface with 5GC (i.e. EP_N6).

It has no attributes defined.";

reference "TS 28.541";

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-05-15 {

description "initial revision";

}

grouping DNFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

}

augment "/me3gpp:ManagedElement" {

list DNFunction {

description "5G Core DN Function";

reference "TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses DNFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## H.5.7	module _3gpp-5gc-nrm-ep@2019-11-18.yang

module _3gpp-5gc-nrm-ep {

yang-version 1.1;

namespace "urn:3gpp:tsg:sa5:nrm:_3gpp-5gc-nrm-ep";

prefix "cep3gpp";

import _3gpp-common-ep-rp { prefix eprp3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-5gc-nrm-affunction { prefix af3gpp; }

import _3gpp-5gc-nrm-amffunction { prefix amf3gpp; }

import _3gpp-5gc-nrm-ausffunction { prefix ausf3gpp; }

import _3gpp-5gc-nrm-dnfunction { prefix dn3gpp; }

import _3gpp-5gc-nrm-lmffunction { prefix lmf3gpp; }

import _3gpp-5gc-nrm-n3iwffunction { prefix n3iwf3gpp; }

import _3gpp-5gc-nrm-ngeirfunction { prefix ngeir3gpp; }

import _3gpp-5gc-nrm-nrffunction { prefix nrf3gpp; }

import _3gpp-5gc-nrm-nssffunction { prefix nssf3gpp; }

import _3gpp-5gc-nrm-pcffunction { prefix pcf3gpp; }

import _3gpp-5gc-nrm-seppfunction { prefix sepp3gpp; }

import _3gpp-5gc-nrm-smffunction { prefix smf3gpp; }

import _3gpp-5gc-nrm-smsffunction { prefix smsf3gpp; }

import _3gpp-5gc-nrm-udmfunction { prefix udm3gpp; }

import _3gpp-5gc-nrm-upffunction { prefix upf3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import ietf-inet-types { prefix inet; }

organization "3GPP SA5";

description "Defines the YANG mapping of the 5GC related endpoint

Information Object Classes (IOCs) that are part of the 5G Core

Network Resource Model.";

reference "TS 28.541";

revision 2019-11-18 {

description "Ericsson refactoring.";

}

revision 2018-07-31 {

description "Initial revision";

}

grouping EP_N2Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N3Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N4Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N5Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N6Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N7Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N8Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N9Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N10Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N11Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N12Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N13Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N14Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N15Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N16Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N17Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N20Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N21Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N22Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N26Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N27Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N31Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N32Grp {

uses eprp3gpp:EP_Common;

container remotePlmnId {

description "PLMN Identifiers of the remote sepp.

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

uses types3gpp:PLMNId;

}

leaf remoteSeppAddress {

description "The host address of the SEPP.";

type inet:host;

}

leaf remoteSeppId {

type uint16;

}

leaf n32cParas {

type string;

}

leaf n32fPolicy {

type string;

}

leaf withIPX {

type boolean;

}

}

grouping EP_S5CGrp {

uses eprp3gpp:EP_Common;

}

grouping EP_S5UGrp {

uses eprp3gpp:EP_Common;

}

grouping EP_RxGrp {

uses eprp3gpp:EP_Common;

}

grouping EP_MAP_SMSCGrp {

uses eprp3gpp:EP_Common;

}

grouping EP_NLSGrp {

uses eprp3gpp:EP_Common;

}

grouping EP_NLGGrp {

uses eprp3gpp:EP_Common;

}

grouping EP_SBI_IPXGrp {

uses eprp3gpp:EP_Common;

leaf-list sBIService {

min-elements 1;

config false;

type string;

}

}

augment "/me3gpp:ManagedElement/af3gpp:AFFunction" {

list EP_N6 {

description "Represents the EP_N6 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N6Grp;

}

}

list EP_Rx {

description "Represents the EP_Rx IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_RxGrp;

}

}

}

augment "/me3gpp:ManagedElement/amf3gpp:AMFFunction" {

list EP_N2 {

description "Represents the EP_N2 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N2Grp;

}

}

list EP_N8 {

description "Represents the EP_N8 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N8Grp;

}

}

list EP_N11 {

description "Represents the EP_N11 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N11Grp;

}

}

list EP_N12 {

description "Represents the EP_N12 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N12Grp;

}

}

list EP_N14 {

description "Represents the EP_N14 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N14Grp;

}

}

list EP_N15 {

description "Represents the EP_N15 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N15Grp;

}

}

list EP_N17 {

description "Represents the EP_N17 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N17Grp;

}

}

list EP_N20 {

description "Represents the EP_N20 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N20Grp;

}

}

list EP_N22 {

description "Represents the EP_N22 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N22Grp;

}

}

list EP_N26 {

description "Represents the EP_N26 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N26Grp;

}

}

list EP_NLS {

description "Represents the EP_NLS IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_NLSGrp;

}

}

list EP_NLG {

description "Represents the EP_NLG IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_NLGGrp;

}

}

}

augment "/me3gpp:ManagedElement/ausf3gpp:AUSFFunction" {

list EP_N12 {

description "Represents the EP_N12 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N12Grp;

}

}

list EP_N13 {

description "Represents the EP_N13 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N13Grp;

}

}

}

augment "/me3gpp:ManagedElement/dn3gpp:DNFunction" {

list EP_N6 {

description "Represents the EP_N6 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N6Grp;

}

}

}

augment "/me3gpp:ManagedElement/lmf3gpp:LMFFunction" {

list EP_NLS {

description "Represents the EP_NLS IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_NLSGrp;

}

}

}

augment "/me3gpp:ManagedElement/n3iwf3gpp:N3IWFFunction" {

list EP_N2 {

description "Represents the EP_N2 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N2Grp;

}

}

list EP_N3 {

description "Represents the EP_N3 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N3Grp;

}

}

}

augment "/me3gpp:ManagedElement/ngeir3gpp:NGEIRFunction" {

list EP_N17 {

description "Represents the EP_N17 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N17Grp;

}

}

}

augment "/me3gpp:ManagedElement/nrf3gpp:NRFFunction" {

list EP_N27 {

description "Represents the EP_N27 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N26Grp;

}

}

}

augment "/me3gpp:ManagedElement/nssf3gpp:NSSFFunction" {

list EP_N22 {

description "Represents the EP_N22 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N22Grp;

}

}

list EP_N31 {

description "Represents the EP_N31 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N31Grp;

}

}

}

augment "/me3gpp:ManagedElement/pcf3gpp:PCFFunction" {

list EP_N5 {

description "Represents the EP_N5 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N5Grp;

}

}

list EP_N7 {

description "Represents the EP_N7 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N7Grp;

}

}

list EP_N15 {

description "Represents the EP_N15 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N15Grp;

}

}

list EP_N16 {

description "Represents the EP_N16 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N16Grp;

}

}

list EP_Rx {

description "Represents the EP_Rx IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_RxGrp;

}

}

}

augment "/me3gpp:ManagedElement/sepp3gpp:SEPPFunction" {

list EP_N32 {

description "Represents the EP_N32 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N32Grp;

}

}

}

augment "/me3gpp:ManagedElement/smsf3gpp:SMSFFunction" {

list EP_N20 {

description "Represents the EP_20 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N20Grp;

}

}

list EP_N21 {

description "Represents the EP_N21 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N21Grp;

}

}

list EP_MAP_SMSC {

description "Represents the EP_MAP_SMSC IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_MAP_SMSCGrp;

}

}

}

augment "/me3gpp:ManagedElement/smf3gpp:SMFFunction" {

list EP_N4 {

description "Represents the EP_N4 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N4Grp;

}

}

list EP_N7 {

description "Represents the EP_N7 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N7Grp;

}

}

list EP_N10 {

description "Represents the EP_N10 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N10Grp;

}

}

list EP_N11 {

description "Represents the EP_N11 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N11Grp;

}

}

list EP_N16 {

description "Represents the EP_N16 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N16Grp;

}

}

list EP_S5C {

description "Represents the EP_S5C IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_S5CGrp;

}

}

}

augment "/me3gpp:ManagedElement/udm3gpp:UDMFunction" {

list EP_N8 {

description "Represents the EP_N8 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N8Grp;

}

}

list EP_N10 {

description "Represents the EP_N10 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N10Grp;

}

}

list EP_N13 {

description "Represents the EP_N13 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N13Grp;

}

}

}

augment "/me3gpp:ManagedElement/upf3gpp:UPFFunction" {

list EP_N4 {

description "Represents the EP_N4 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N4Grp;

}

}

list EP_N3 {

description "Represents the EP_N3 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N3Grp;

}

}

list EP_N9 {

description "Represents the EP_N9 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N9Grp;

}

}

list EP_S5U {

description "Represents the EP_S5U IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_S5UGrp;

}

}

list EP_EP_N6 {

description "Represents the EP_N6 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N6Grp;

}

}

}

}

## H.5.8	module _3gpp-5gc-nrm-externalnrffunction@2019-10-28.yang

module _3gpp-5gc-nrm-externalnrffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-externalnrffunction;

prefix extnrf3gpp;

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

description "This IOC represents external NRF function controlled by another management domain.";

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-11 {

description "Ericsson refactoring.";

}

grouping ExternalNRFFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

}

augment "/subnet3gpp:SubNetwork" {

list ExternalNRFFunction {

description "5G Core External NRF Function";

reference "TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalNRFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;    }

}

}

### H.5.9	module _3gpp-5gc-nrm-externalnssffunction@2019-10-28.yang

module _3gpp-5gc-nrm-externalnssffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-externalnssffunction;

prefix extnssf3gpp;

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

description "This IOC represents external NSSF function controlled by another management domain.";

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-11 {

description "Ericsson refactoring.";

}

grouping ExternalNSSFFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

}

augment "/subnet3gpp:SubNetwork" {

list ExternalNSSFFunction {

description "5G Core External NSSF Function";

reference "TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalNSSFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## H.5.10	module _3gpp-5gc-nrm-lmffunction@2019-10-25.yang

module _3gpp-5gc-nrm-lmffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-lmffunction;

prefix lmf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

description "This IOC represents the LMF function defined in TS 23.501.";

reference "TS 28.541";

revision 2019-10-25 { reference "S5-194457 S5193518"; }

revision 2019-05-15 {

description "initial revision";

reference "Based on

TS 28.541 V15.X.XX";

}

grouping LMFFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

list managedNFProfile {

key idx;

min-elements 1;

uses types3gpp:ManagedNFProfile;

}

list commModelList {

min-elements 1;

key "groupId";

uses types5g3gpp:CommModel;

}

}

augment "/me3gpp:ManagedElement" {

list LMFFunction {

description "5G Core LMF Function";

reference "TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses LMFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## H.5.11	module _3gpp-5gc-nrm-n3iwffunction@2019-10-28.yang

module _3gpp-5gc-nrm-n3iwffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-n3iwffunction;

prefix n3iwf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

description "This IOC represents the N3IWF function which is used to enable non-3GPP

access networks connected to the 5GC. For more information about the N3IWF, see TS 23.501.";

reference "TS 28.541";

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-05-22 {

description "initial revision";

}

grouping N3IWFFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

list commModelList {

min-elements 1;

key "groupId";

uses types5g3gpp:CommModel;

}

}

augment "/me3gpp:ManagedElement" {

list N3IWFFunction {

description "5G Core N3IWF Function";

reference "TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses N3IWFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## H.5.12	module _3gpp-5gc-nrm-nfprofile@2019-06-17.yang

module _3gpp-5gc-nrm-nfprofile {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-nfprofile;

prefix nfp3gpp;

import _3gpp-common-yang-types { prefix types3gpp; }

import ietf-inet-types { prefix inet; }

import ietf-yang-types { prefix yang; }

import _3gpp-5gc-nrm-nfservice { prefix nfs3gpp; }

organization "3gpp SA5";

description "NF profile class.";

reference "TS 29.510";

revision 2019-06-17 {

description "initial revision";

}

grouping NFProfileGrp {

leaf nfInstanceID {

description "String uniquely identifying a NF instance.";

mandatory true;

type string;

}

leaf nfType {

description "Type of Network Function.";

mandatory true;

type types3gpp:NfType;

}

leaf nfStatus {

description "Status of the NF Instance.";

mandatory true;

type NFStatus;

}

leaf heartBeatTimer {

description "Time in seconds expected between 2 consecutive heart-beat messages from

an NF Instance to the NRF. It may be included in the registration request.

When present in the request it shall contain the heartbeat time proposed by the NF service consumer.";

//conditional support

type uint16;

}

list plmnList {

description "PLMN(s) of the Network Function.

This IE shall be present if this information is available for the NF.

If not provided, PLMN ID(s) of the PLMN of the NRF are assumed for the NF.";

//conditional support

min-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

list sNssais { //is the key unique

description "S-NSSAIs of the Network Function. If not provided, the NF can serve any S-NSSAI.

When present this IE represents the list of S-NSSAIs supported in all the PLMNs listed in the plmnList IE.";

min-elements 1;

//optional support

key "sst sd";

uses Snssai;

}

list perPlmnSnssaiList {

description "This IE may be included when the list of S-NSSAIs supported by the NF for each PLMN it is supporting is different.

When present, this IE shall include the S-NSSAIs supported by the Network Function

for each PLMN supported by the Network Function. When present, this IE shall override sNssais IE.";

min-elements 1;

//optional support

key idx; //no obvious leaf to use as a key

leaf idx { type uint32; }

uses PlmnSnssai;

}

leaf-list nsiList {

description "NSI identities of the Network Function.

If not provided, the NF can serve any NSI.";

//optional support

min-elements 1;

type string;

}

leaf fqdn {

description "FQDN of the Network Function. For AMF, the FQDN registered with the NRF

shall be that of the AMF Name.";

//conditional support

type inet:domain-name;

}

leaf interPlmnFqdn {

description "If the NF needs to be discoverable by other NFs in a different PLMN,

then an FQDN that is used for inter-PLMN routing is specified.";

//conditional support

type inet:domain-name;

}

leaf-list ipv4Addresses {

description "IPv4 address(es) of the Network Function.";

min-elements 1;

//conditional support

type inet:ipv4-address;

}

leaf-list ipv6Addresses {

description "IPv6 address(es) of the Network Function.";

min-elements 1;

//conditional support

type inet:ipv6-address;

}

list allowedPlmns {

description "PLMNs allowed to access the NF instance.

If not provided, any PLMN is allowed to access the NF.";

min-elements 1;

//optional support

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf-list allowedNfTypes {

description "Type of the NFs allowed to access the NF instance.

If not provided, any NF type is allowed to access the NF.";

min-elements 1;

//optional support

type types3gpp:NfType;

}

leaf-list allowedNfDomains {

description "Pattern representing the NF domain names allowed to access the NF instance.

If not provided, any NF domain is allowed to access the NF.";

min-elements 1;

//optional support

type string;

}

list allowedNssais { //is the key unique

description "S-NSSAI of the allowed slices to access the NF instance.

If not provided, any slice is allowed to access the NF.";

min-elements 1;

//optional support

key "sst sd";

uses Snssai;

}

leaf priority {

description "Priority (relative to other NFs of the same type) in the range of 0-65535, to be used for NF selection;

lower values indicate a higher priority. If priority is also present in the nfServiceList parameters,

those will have precedence over this value. The NRF may overwrite the received priority value when exposing

an NFProfile with the Nnrf_NFDiscovery service.";

//optional support

type uint16;

}

leaf capacity {

description "Static capacity information in the range of 0-65535, expressed as a weight

relative to other NF instances of the same type; if capacity is also present

in the nfServiceList parameters, those will have precedence over this value.";

//optional support

type uint16;

}

leaf load {

description "Dynamic load information, ranged from 0 to 100, indicates the current load percentage of the NF.";

//optional support

type types3gpp:Load;

}

leaf locality {

description "Operator defined information about the location of the NF instance (e.g. geographic location, data center).";

//optional support

type string;

}

grouping udrInfo {

//optional support

leaf groupId {

description "Identity of the UDR group that is served by the UDR instance.

If not provided, the UDR instance does not pertain to any UDR group.";

//optional support

type string;

}

list supiRanges {

description "List of ranges of SUPI's whose profile data is available in the UDR instance.";

key "start end pattern";

min-elements 1;

//optional support

uses SupiRange;

}

list gpsiRanges {

description "List of ranges of GPSIs whose profile data is available in the UDR instance.";

key "start end pattern";

min-elements 1;

//optional support

uses IdentityRange;

}

list externalGroupIdentifiersRanges {

description "List of ranges of external groups whose profile data is available in the UDR instance.";

key "start end pattern";

min-elements 1;

//optional support

uses IdentityRange;

}

leaf-list supportedDataSets {

description "List of supported data sets in the UDR instance.

If not provided, the UDR supports all data sets.";

min-elements 1;

//optional support

type DataSetId;

}

}

grouping udmInfo {

//optional support

leaf groupId {

description "Identity of the UDM group that is served by the UDM instance.

If not provided, the UDM instance does not pertain to any UDM group.";

//optional support

type string;

}

list supiRanges {

description "List of ranges of SUPI's whose profile data is available in the UDM instance.";

key "start end pattern";

min-elements 1;

//optional support

uses SupiRange;

}

list gpsiRanges {

description "List of ranges of GPSIs whose profile data is available in the UDM instance.";

key "start end pattern";

min-elements 1;

//optional support

uses IdentityRange;

}

list externalGroupIdentifiersRanges {

description "List of ranges of external groups whose profile data is available in the UDM instance.";

key "start end pattern";

min-elements 1;

//optional support

uses IdentityRange;

}

leaf-list routingIndicators {

description "List of Routing Indicator information that allows to route network signalling with SUCI

to the UDM instance. If not provided, the UDM can serve any Routing Indicator.

Pattern: '^[0-9]{1,4}$'.";

//optional support

min-elements 1;

type string;

}

}

grouping ausfInfo {

//optional support

leaf groupId {

description "Identity of the AUSF group. If not provided, the AUSF instance does not pertain to any AUSF group.";

//optional support

type string;

}

list supiRanges {

description "List of ranges of SUPIs that can be served by the AUSF instance. If not provided, the AUSF can serve any SUPI.";

key "start end pattern";

min-elements 1;

//optional support

uses SupiRange;

}

leaf-list routingIndicators {

description "List of Routing Indicator information that allows to route network signalling with SUCI

to the AUSF instance. If not provided, the AUSF can serve any Routing Indicator.

Pattern: '^[0-9]{1,4}$'.";

//optional support

min-elements 1;

type string;

}

}

grouping amfInfo {

//optional support

leaf amfRegionId {

description "AMF region identifier";

type string;

}

leaf amfSetId {

description "AMF set identifier";

type string;

}

list guamiList {

description "List of supported GUAMIs.";

key idx; //no obvious leaf to use as a key

leaf idx { type uint32; }

min-elements 1;

uses Guami;

}

list taiList {

description "The list of TAIs the AMF can serve. It may contain the non-3GPP access TAI.

The absence of this attribute and the taiRangeList attribute indicate that

the AMF can be selected for any TAI in the serving network.";

key idx; //no obvious leaf to use as a key

leaf idx { type uint32; }

//optional support

min-elements 1;

uses Tai;

}

list taiRangeList {

description "The range of TAIs the AMF can serve. The absence of this attribute and the taiList

attribute indicate that the AMF can be selected for any TAI in the serving network.";

//optional support

min-elements 1;

key idx; //no obvious leaf to use as a key

leaf idx { type uint32; }

uses TaiRange;

}

list backupInfoAmfFailure {

description "List of GUAMIs for which the AMF acts as a backup for AMF failure.";

key idx; //no obvious leaf to use as a key

leaf idx { type uint32; }

//optional support

min-elements 1;

uses Guami;

}

list backupInfoAmfRemoval {

description "List of GUAMIs for which the AMF acts as a backup for planned AMF removal.";

key idx; //no obvious leaf to use as a key

leaf idx { type uint32; }

//optional support

min-elements 1;

uses Guami;

}

list n2InterfaceAmfInfo {

description "N2 interface information of the AMF. This information needs not be sent in NF Discovery responses.

It may be used by the NRF to update the DNS for AMF discovery by the 5G Access Network.";

//optional support

max-elements 1;

key idx; //no obvious leaf to use as a key

leaf idx { type uint32; }

uses N2InterfaceAmfInfo;

}

}

grouping smfInfo {

//optional support

list sNssaiSmfInfoList {

description "List of parameters supported by the SMF per S-NSSAI.";

min-elements 1;

key idx; //no obvious leaf to use as a key

leaf idx { type uint32; }

uses sNssaiSmfInfoItem;

}

list taiList {

description "The list of TAIs the SMF can serve. It may contain the non-3GPP access TAI.

The absence of this attribute and the taiRangeList attribute indicate that

the SMF can be selected for any TAI in the serving network.";

key idx; //no obvious leaf to use as a key

leaf idx { type uint32; }

//optional support

min-elements 1;

uses Tai;

}

list taiRangeList {

description "The range of TAIs the SMF can serve. The absence of this attribute and the taiList

attribute indicate that the SMF can be selected for any TAI in the serving network.";

//optional support

min-elements 1;

key idx; //no obvious leaf to use as a key

leaf idx { type uint32; }

uses TaiRange;

}

leaf pgwFqdn {

description "The FQDN of the PGW if the SMF is a combined SMF/PGW-C.";

//optional support

type inet:domain-name;

}

leaf-list accessType {

description "If included, this IE shall contain the access type (3GPP_ACCESS and/or NON_3GPP_ACCESS) supported by the SMF.

If not included, it shall be assumed the both access types are supported.";

//conditional support

min-elements 1;

max-elements 2;

type AccessType;

}

}

grouping upfInfo {

//optional support

list sNssaiUpfInfoList {

description "List of parameters supported by the UPF per S-NSSAI.";

min-elements 1;

key idx; //no obvious leaf to use as a key

leaf idx { type uint32; }

uses SnssaiUpfInfoItem;

}

leaf-list smfServingArea {

description "The SMF service area(s) the UPF can serve.

If not provided, the UPF can serve any SMF service area.";

//optional support

min-elements 1;

type string;

}

list interfaceUpfInfo {

description "List of User Plane interfaces configured on the UPF. When this IE is provided in the NF Discovery response,

the NF Service Consumer (e.g. SMF) may use this information for UPF selection.";

key idx; //no obvious leaf to use as a key

leaf idx { type uint32; }

//optional support

min-elements 1;

uses InterfaceUpfInfoItem;

}

leaf iwkEpsInd {

description "Indicates whether interworking with EPS is supported by the UPF.

true: Supported

false (default): Not Supported";

//optional support

type boolean;

}

leaf-list pduSessionTypes {

description "List of PDU session type(s) supported by the UPF. The absence of this attribute indicates that the UPF can be selected

for any PDU session type.";

//optional support

min-elements 1;

type PduSessionType;

}

}

grouping pcfInfo {

//optional support

leaf-list dnnList {

description "DNNs supported by the PCF.

If not provided, the PCF can serve any DNN.";

//optional support

min-elements 1;

type string;

}

list supiRanges {

description "List of ranges of SUPIs that can be served by the PCF instance. If not provided, the PCF can serve any SUPI.";

key "start end pattern";

min-elements 1;

//optional support

uses SupiRange;

}

leaf rxDiamHost {

description "This IE shall be present if the PCF supports Rx interface.

When present, this IE shall indicate the Diameter host of the Rx interface for the PCF.

Pattern: '^([A-Za-z0-9]+(-[A-Za-z0-9]+).)+[a-z]{2,}$'.";

//conditional support

type string;

}

leaf rxDiamRealm {

description "This IE shall be present if the PCF supports Rx interface.

When present, this IE shall indicate the Diameter realm of the Rx interface for the PCF.

Pattern: '^([A-Za-z0-9]+(-[A-Za-z0-9]+).)+[a-z]{2,}$'.";

//conditional support

type string;

}

}

grouping bsfInfo {

//optional support

list ipv4AddressRanges {

description "List of ranges of IPv4 addresses handled by BSF.

If not provided, the BSF can serve any IPv4 address.";

//optional support

key "start end";

uses types3gpp:Ipv4AddressRange;

}

leaf-list dnnList {

description "List of DNNs handled by the BSF

If not provided, the BSF can serve any DNN.";

//optional support

min-elements 1;

type string;

}

leaf-list ipDomainList {

description "List of IPv4 address domains, as described in clause 6.2 of TS 29.513, handled by the BSF.

If not provided, the BSF can serve any IP domain.";

//optional support

min-elements 1;

type string;

}

list ipv6PrefixRanges {

description "List of ranges of IPv6 prefixes handled by the BSF.

If not provided, the BSF can serve any IPv6 prefix.";

//optional support

key "start end";

uses types3gpp:Ipv6PrefixRange;

}

}

grouping chfInfo {

//optional support

list supiRangeList {

description "List of ranges of SUPIs that can be served by the CHF instance. If not provided, the CHF can serve any SUPI.";

key "start end pattern";

min-elements 1;

//optional support

uses SupiRange;

}

list gpsiRangeList {

description "List of ranges of GPSI that can be served by the CHF instance. If not provided, the CHF can serve any GPSI.";

key "start end pattern";

min-elements 1;

//optional support

uses IdentityRange;

}

list plmnRangeList {

description "List of ranges of PLMNs (including the PLMN IDs of the CHF instance) that can be served by the CHF instance.

If not provided, the CHF can serve any PLMN.";

min-elements 1;

//optional support

key "mcc mnc";

uses types3gpp:PLMNId;

}

}

grouping nrfInfoGrp {

//optional support

list servedUdrInfo {

description "This attribute contains all the udrInfo attributes locally configured in the NRF or the NRF received during NF registration.";

//optional support

key nfInstanceID;

leaf nfInstanceID {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses udrInfo;

}

list servedUdmInfo {

description "This attribute contains all the udmInfo attributes locally configured in the NRF or the NRF received during NF registration.";

//optional support

key nfInstanceID;

leaf nfInstanceID {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses udmInfo;

}

list servedAusfInfo {

description "This attribute contains all the ausfInfo attributes locally configured in the NRF or the NRF received during NF registration.";

//optional support

key nfInstanceID;

leaf nfInstanceID {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses ausfInfo;

}

list servedAmfInfo {

description "This attribute contains all the amfInfo attributes locally configured in the NRF or the NRF received during NF registration.";

//optional support

key nfInstanceID;

leaf nfInstanceID {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses amfInfo;

}

list servedSmfInfo {

description "This attribute contains all the smfInfo attributes locally configured in the NRF or the NRF received during NF registration.";

//optional support

key nfInstanceID;

leaf nfInstanceID {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses smfInfo;

}

list servedUpfInfo {

description "This attribute contains all the upfInfo attributes locally configured in the NRF or the NRF received during NF registration.";

//optional support

key nfInstanceID;

leaf nfInstanceID {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses upfInfo;

}

list servedPcfInfo {

description "This attribute contains all the pcfInfo attributes locally configured in the NRF or the NRF received during NF registration.";

//optional support

key nfInstanceID;

leaf nfInstanceID {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses pcfInfo;

}

list servedBsfInfo {

description "This attribute contains all the bsfInfo attributes locally configured in the NRF or the NRF received during NF registration.";

//optional support

key nfInstanceID;

leaf nfInstanceID {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses bsfInfo;

}

list servedChfInfo {

description "This attribute contains all the bsfInfo attributes locally configured in the NRF or the NRF received during NF registration.";

//optional support

key nfInstanceID;

leaf nfInstanceID {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses chfInfo;

}

}

list nrfInfo {

key idx; //no obvious leaf to use as a key

leaf idx { type uint32; }

max-elements 1;

uses nrfInfoGrp;

}

leaf customInfo {

description "Specific data for custom Network Functions.";

type string;

}

leaf recoveryTime {

description "Timestamp when the NF was (re)started.";

//optional support

type yang:date-and-time;

}

leaf nfServicePersistence {

description "If present, and set to true, it indicates that the different service instances of a same NF Service in this NF instance,

supporting a same API version, are capable to persist their resource state in shared storage and therefore these resources

are available after a new NF service instance supporting the same API version is selected by a NF Service Consumer (see TS 23.527).

Otherwise, it indicates that the NF Service Instances of a same NF Service are not capable to share resource state inside the NF Instance.";

//optional support

type boolean;

}

list nfServices {

description "List of NF Service Instances. It shall include the services produced by the NF that can be discovered by other NFs.";

key serviceInstanceID;

//optional support

min-elements 1;

uses nfs3gpp:NFServiceGrp;

}

leaf nfProfileChangesSupportInd {

description "NF Profile Changes Support Indicator. This IE may be present in the NFRegister or NFUpdate (NF Profile Complete Replacement) request and shall be absent in the response.

true: the NF Service Consumer supports receiving NF Profile Changes in the response.

false (default): the NF Service Consumer does not support receiving NF Profile Changes in the response.";

//optional support

type boolean;

}

leaf nfProfileChangesInd {

description "NF Profile Changes Indicator. This IE shall be absent in the request to the NRF and may be included by the NRF in NFRegister or NFUpdate (NF Profile Complete Replacement) response.

true: the NF Profile contains NF Profile changes.

false (default): complete NF Profile.";

//optional support

type boolean;

}

list defaultNotificationSubscriptions {

description "Notification endpoints for different notification types.";

key notificationType;

//optional support

min-elements 1;

uses types3gpp:DefaultNotificationSubscription;

}

}

typedef NFStatus {

type enumeration {

enum REGISTERED;

enum SUSPENDED;

}

}

typedef DataSetId {

type enumeration {

enum SUBSCRIPTION;

enum POLICY;

enum EXPOSURE;

enum APPLICATION;

}

}

grouping SupiRange {

leaf start {

description "First value identifying the start of a SUPI range. To be used when the range of SUPI's can be represented as a numeric range (e.g., IMSI ranges).";

type string {

pattern '^[0-9]+$';

}

}

leaf end {

description "Last value identifying the end of a SUPI range. To be used when the range of SUPI's can be represented as a numeric range (e.g. IMSI ranges).";

type string {

pattern '^[0-9]+$';

}

}

leaf pattern {

description "Pattern representing the set of SUPI's belonging to this range.

A SUPI value is considered part of the range if and only if the SUPI string fully matches the regular expression.";

type string;

}

}

grouping IdentityRange {

leaf start {

description "First value identifying the start of an identity range. To be used when the range of identities can be represented as a numeric range (e.g., MSISDN ranges).";

type string {

pattern '^[0-9]+$';

}

}

leaf end {

description "Last value identifying the end of an identity range. To be used when the range of identities can be represented as a numeric range (e.g. MSISDN ranges).";

type string {

pattern '^[0-9]+$';

}

}

leaf pattern {

description "Pattern representing the set of identities belonging to this range.

An identity value is considered part of the range if and only if the identity string fully matches the regular expression.";

type string;

}

}

grouping TacRange {

leaf start {

description "First value identifying the start of a TAC range, to be used when the range of TAC's can be represented as a hexadecimal range (e.g., TAC ranges).";

type string {

pattern '^([A-Fa-f0-9]{4}|[A-Fa-f0-9]{6}$)';

}

}

leaf end {

description "Last value identifying the end of a TAC range, to be used when the range of TAC's can be represented as a hexadecimal range (e.g. TAC ranges).";

type string {

pattern '^([A-Fa-f0-9]{4}|[A-Fa-f0-9]{6})$';

}

}

leaf pattern {

description "Pattern representing the set of TAC's belonging to this range.";

type string;

}

}

grouping SnssaiUpfInfoItem {

list sNssai { //is the key unique

description "Supported S-NSSAI.";

min-elements 1;

max-elements 1;

key "sst sd";

uses Snssai;

}

list dnnUpfInfoList {

description "List of parameters supported by the UPF per DNN.";

min-elements 1;

key dnn;

uses DnnUpfInfoItem;

}

}

grouping DnnUpfInfoItem {

leaf dnn {

description "String representing a Data Network.";

mandatory true;

type string;

}

leaf-list dnaiList {

description "List of Data network access identifiers supported by the UPF for this DNN.

The absence of this attribute indicates that the UPF can be selected for this DNN for any DNAI.";

min-elements 1;

type string; //dnai is the type but its only a string with desc: DNAI (Data network access identifier), is this needed as its own typedef or string is ok

}

leaf-list pduSessionTypes {

description "List of PDU session type(s) supported by the UPF for a specific DNN.";

min-elements 1;

type PduSessionType;

}

}

grouping Snssai {

leaf sst {

description "Unsigned integer, within the range 0 to 255, representing the Slice/Service Type.

It indicates the expected Network Slice behaviour in terms of features and services.";

mandatory true;

type uint32;

}

leaf sd {

description "3-octet string, representing the Slice Differentiator, in hexadecimal representation.";

//optional

type string {

pattern '^[A-Fa-f0-9]{6}$';

}

}

reference "TS 29.571";

}

typedef PduSessionType {

type enumeration {

enum IPV4;

enum IPV6;

enum IPV4V6;

enum UNSTRUCTURED;

enum ETHERNET;

}

}

grouping Guami {

list plmnId {

description "PLMN Identity.";

min-elements 1;

max-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

list amfId {

description "AMF Identity.";

min-elements 1;

max-elements 1;

key "amfRegionId amfSetId amfPointer";

uses types3gpp:AmfIdentifier;

}

}

grouping Tai {

list plmnId {

description "PLMN Identity.";

min-elements 1;

max-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf tac { type types3gpp:Tac; }

}

grouping InterfaceUpfInfoItem {

leaf interfaceType {

description "User Plane interface type.";

mandatory true;

type UPInterfaceType;

}

////At least one of the addressing parameters (ipv4address, ipv6adress or endpointFqdn) shall be included in the InterfaceUpfInfoItem.

choice address {

case ipv4EndpointAddresses {

leaf-list ipv4EndpointAddresses {

description "Available endpoint IPv4 address(es) of the User Plane interface.";

//conditional support

min-elements 1;

type inet:ipv4-address;

}

}

case ipv6EndpointAddresses {

leaf-list ipv6EndpointAddresses {

description "Available endpoint IPv6 address(es) of the User Plane interface.";

//conditional support

min-elements 1;

type inet:ipv6-address;

}

}

case endpointFqdn {

leaf endpointFqdn {

description "FQDN of available endpoint of the User Plane interface.";

//conditional support

type inet:domain-name;

}

}

}

leaf networkInstance {

description "Network Instance associated to the User Plane interface.";

//optional support

type string;

}

}

typedef UPInterfaceType {

type enumeration {

enum N3;

enum N6;

enum N9;

}

}

grouping TaiRange {

list plmnId {

description "PLMN ID related to the TacRange.";

min-elements 1;

max-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

list tacRangeList { //is this key unique

description "The range of the TACs.";

min-elements 1;

key "start end";

uses TacRange;

}

}

typedef AccessType {

type enumeration {

enum 3GPP_ACCESS;

enum NON_3GPP_ACCESS;

}

}

grouping N2InterfaceAmfInfo {

//At least one of the addressing parameters (ipv4address or ipv6adress) shall be included.

choice address {

case ipv4EndpointAddress {

leaf-list ipv4EndpointAddress {

description "Available AMF endpoint IPv4 address(es) for N2.";

//conditional support

min-elements 1;

type inet:ipv4-address;

}

}

case ipv6EndpointAddress {

leaf-list ipv6EndpointAddress {

description "Available AMF endpoint IPv6 address(es) for N2.";

//conditional support

min-elements 1;

type inet:ipv6-address;

}

}

}

leaf amfName {

description "AMF name.";

type string;

}

}

grouping sNssaiSmfInfoItem {

list sNssai { //is the key unique

description "Supported S-NSSAI.";

min-elements 1;

max-elements 1;

key "sst sd";

uses Snssai;

}

list dnnSmfInfoList { //is the key unique

description "List of parameters supported by the SMF per DNN.";

min-elements 1;

key dnn;

uses DnnSmfInfoItem;

}

}

grouping DnnSmfInfoItem {

leaf dnn {

description "Supported DNN.";

mandatory true;

type string;

}

}

grouping PlmnSnssai {

list plmnId {

description "PLMN ID for which list of supported S-NSSAI(s) is provided.";

min-elements 1;

max-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

list sNssaiList { //is the key unique

description "The specific list of S-NSSAIs supported by the given PLMN.";

min-elements 1;

key "sst sd";

uses Snssai;

}

}

}

## H.5.13	module _3gpp-5gc-nrm-nfservice@2019-06-17.yang

module _3gpp-5gc-nrm-nfservice {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-nfservice;

prefix nfs3gpp;

import _3gpp-common-yang-types { prefix types3gpp; }

import ietf-yang-types { prefix yang; }

import ietf-inet-types { prefix inet; }

organization "3gpp SA5";

description "NF service class.";

reference "TS 29.510";

revision 2019-06-17 {

description "initial revision";

}

grouping NFServiceGrp {

leaf serviceInstanceID {

description "Unique ID of the service instance within a given NF Instance.";

mandatory true;

type string;

}

leaf serviceName {

description "Name of the service instance (e.g. 'nudm-sdm').";

mandatory true;

type ServiceName;

}

list versions { //check in review if key is ok (unique)

description "API versions supported by the NF Service and if available, the corresponding retirement date of the NF Service.";

min-elements 1;

key "apiVersionInUri apiFullVersion";

uses NFServiceVersion;

}

leaf scheme {

description "URI scheme (e.g. 'http', 'https').";

mandatory true;

type UriScheme;

}

leaf nfServiceStatus {

description "Status of the NF Service Instance.";

mandatory true;

type NFServiceStatus;

}

leaf fqdn {

description "FQDN of the NF Service Instance.";

//optional support

type inet:domain-name;

}

leaf interPlmnFqdn {

description "If the NF service needs to be discoverable by other NFs in a different PLMN,

then an FQDN that is used for inter PLMN routing.";

//optional support

type inet:domain-name;

}

list ipEndPoints {

description "IP address(es) and port information of the Network Function (including IPv4 and/or IPv6 address)

where the service is listening for incoming service requests.";

//optional support

key idx;

leaf idx {

type string;

}

min-elements 1;

uses ipEndPoint;

}

leaf apiPrefix {

description "Optional path segment(s) used to construct the {apiRoot} variable of the different API URIs.";

//optional support

type string;

}

list defaultNotificationSubscriptions {

description "Notification endpoints for different notification types.";

key notificationType;

//optional support

min-elements 1;

uses types3gpp:DefaultNotificationSubscription;

}

list allowedPlmns {

description "PLMNs allowed to access the service instance.

The absence of this attribute indicates that any PLMN is allowed to access the service instance.";

min-elements 1;

//optional support

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf-list allowedNfTypes {

description "Type of the NFs allowed to access the service instance.

The absence of this attribute indicates that any NF type is allowed to access the service instance.";

min-elements 1;

//optional support

type types3gpp:NfType;

}

leaf-list allowedNfDomains {

description "Pattern representing the NF domain names allowed to access the service instance.";

//optional support

min-elements 1;

type string;

}

leaf-list allowedNssais {

description "S-NSSAI of the allowed slices to access the service instance.

The absence of this attribute indicates that any slice is allowed to access the service instance.";

min-elements 1;

//optional support

type types3gpp:SNssai;

}

leaf priority {

description "Priority (relative to other services of the same type) in the range of 0-65535,

to be used for NF Service selection; lower values indicate a higher priority.";

//optional support

type uint16;

}

leaf capacity {

description "Static capacity information in the range of 0-65535, expressed as a weight relative to other services of the same type.";

//optional support

type uint16;

}

leaf load {

description "Dynamic load information, ranged from 0 to 100, indicates the current load percentage of the NF Service.";

//optional support

type types3gpp:Load;

}

leaf recoveryTime {

description "Timestamp when the NF was (re)started.";

//optional support

type yang:date-and-time;

}

list chfServiceInfo { //is the key unique

description "Specific data for a CHF service instance.";

//optional support

max-elements 1;

key "primaryChfServiceInstance secondaryChfServiceInstance";

uses ChfServiceInfo;

}

leaf supportedFeatures {

description "Supported Features of the NF Service instance.";

//optional support

type SupportedFeatures;

}

}

typedef SupportedFeatures {

type string {

pattern '[A-Fa-f0-9]*';

}

}

grouping ipEndPoint {

choice address {

leaf ipv4Address {

type inet:ipv4-address;

}

leaf ipv6Address {

type inet:ipv6-address;

}

leaf ipv6Prefix {

type inet:ipv6-prefix;

}

}

leaf transport {

type TransportProtocol;

}

leaf port {

type uint16;

}

}

typedef TransportProtocol {

type enumeration {

enum TCP;

enum STCP;

enum UDP;

}

}

grouping NFServiceVersion {

leaf apiVersionInUri {

mandatory true;

type string;

}

leaf apiFullVersion {

mandatory true;

type string;

}

leaf expiry {

//optional to support

type yang:date-and-time;

}

}

typedef ServiceName {

type enumeration {

enum NNRF_NFM;

enum NNRF_DISC;

enum NUDM_SDM;

enum NUDM_UECM;

enum NUDM_UEAU;

enum NUDM_EE;

enum NUDM_PP;

enum NAMF_COMM;

enum NAMF_EVTS;

enum NAMF_MT;

enum NAMF_LOC;

enum NSMF_PDUSESSION;

enum NSMF_EVENT-EXPOSURE;

enum NAUSF_AUTH;

enum NAUSF_SORPROTECTION;

enum NNEF_PFDMANAGEMENT;

enum NPCF_AM-POLICY-CONTROL;

enum NPCF_SMPOLICYCONTROL;

enum NPCF_POLICYAUTHORIZATION;

enum NPCF_BDTPOLICYCONTROL;

enum NPCF_EVENTEXPOSURE;

enum NPCF_UE_POLICY_CONTROL;

enum NSMSF_SMS;

enum NNSSF_NSSELECTION;

enum NNSSF_NSSAIAVAILABILITY;

enum NUDR_DR;

enum NLMF_LOC;

enum N5G_EIR_EIC;

enum NBSF_MANAGEMENT;

enum NCHF_SPENDINGLIMITCONTROL;

enum NCHF_CONVERGEDCHARGING;

enum NNWDAF_EVENTSSUBSCRIPTION;

enum NNWDAF_ANALYTICSINFO;

}

}

typedef UriScheme {

type enumeration {

enum HTTP;

enum HTTPS;

}

}

typedef NFServiceStatus {

type enumeration {

enum REGISTERED;

enum SUSPENDED;

enum UNDISCOVERABLE;

}

}

grouping ChfServiceInfo {

leaf primaryChfServiceInstance {

description "Shall be present if the CHF service instance serves as a secondary CHF instance of another primary CHF service instance.";

//conditional to support

type string;

}

leaf secondaryChfServiceInstance {

description "Shall be present if the CHF service instance serves as a primary CHF instance of another secondary CHF service instance.";

//conditional to support

type string;

}

}

}

## H.5.14	module _3gpp-5gc-nrm-ngeirfunction@2019-10-25.yang

module _3gpp-5gc-nrm-ngeirfunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-ngeirfunction;

prefix ngeir3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

description "This IOC represents the 5G-EIR function in 5GC. For more information about the 5G-EIR, see TS 23.501.";

reference "TS 28.541";

revision 2019-10-25 { reference "S5-194457 S5-195427 S5-193518"; }

revision 2019-05-15 {

description "initial revision";

reference "Based on

TS 28.541 V15.X.XX";

}

grouping NGEIRFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf-list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//optional support

reference "TS 23.003";

type types3gpp:SNssai;

}

list managedNFProfile {

key idx;

min-elements 1;

uses types3gpp:ManagedNFProfile;

}

list commModelList {

min-elements 1;

key "groupId";

uses types5g3gpp:CommModel;

}

}

augment "/me3gpp:ManagedElement" {

list NGEIRFunction {

description "5G Core NGEIR Function";

reference "TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NGEIRFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## H.5.15	module _3gpp-5gc-nrm-nrffunction.yang

module _3gpp-5gc-nrm-nrffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-nrffunction;

prefix nrf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5gc-nrm-nfprofile { prefix nfp3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

description "This IOC represents the NRF function in 5GC.

For more information about the NRF, see TS 23.501 [2].";

reference "TS 28.541";

revision 2020-08-03 { reference "CR-0321"; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-05-15 { description "initial revision"; }

grouping NRFFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based interface.";

type inet:domain-name;

}

leaf-list cNSIIdList {

description "NSI ID. NSI ID is an identifier for identifying the Core

Network part of a Network Slice instance when multiple Network Slice

instances of the same Network Slice are deployed, and there is a need

to differentiate between them in the 5GC, see clause 3.1 of TS 23.501

and clause 6.1.6.2.7 of TS 29.531";

type string;

}

leaf-list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//optional support

reference "TS 23.003";

type types3gpp:SNssai;

}

list nFProfileList {

description "Set of NFProfile(s) to be registered in the NRF instance.";

//optional support

key nfInstanceID;

uses nfp3gpp:NFProfileGrp;

}

}

augment "/me3gpp:ManagedElement" {

list NRFFunction {

description "5G Core NRF Function";

reference "TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NRFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## H.5.16	module _3gpp-5gc-nrm-nssffunction.yang

module _3gpp-5gc-nrm-nssffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-nssffunction;

prefix nssf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

description "This IOC represents the NSSF function in 5GC. For more

information about the NSSF, see TS 23.501.";

reference "TS 28.541";

revision 2020-08-03 { reference "CR-0321"; }

revision 2019-10-25 { reference "S5-194457 S5-195427 S5-193518"; }

revision 2019-05-15 { description "initial revision"; }

grouping NSSFFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least

one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a

Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {      description "The FQDN of the registered NF instance in the service-based

interface.";

type inet:domain-name;

}

leaf-list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

reference "TS 23.003";

type types3gpp:SNssai;

}

leaf-list cNSIIdList {

description "NSI ID. NSI ID is an identifier for identifying the Core

Network part of a Network Slice instance when multiple Network Slice

instances of the same Network Slice are deployed, and there is a need

to differentiate between them in the 5GC, see clause 3.1 of TS 23.501

and clause 6.1.6.2.7 of TS 29.531";

type string;

}

list managedNFProfile {

key idx;

min-elements 1;

uses types3gpp:ManagedNFProfile;

}

}

augment "/me3gpp:ManagedElement" {

list NSSFFunction {

description "5G Core NSSF Function";

reference "TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NSSFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## H.5.17	module _3gpp-5gc-nrm-nwdaffunction@2019-10-25.yang

module _3gpp-5gc-nrm-nwdaffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-nwdaffunction;

prefix nwdaf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

description "This IOC represents the NWDAF function in 5GC. For more information about the NWDAF, see TS 23.501.";

reference "TS 28.541";

revision 2019-10-25 { reference "S5-194457 S5-195427 S5-193518"; }

revision 2019-05-15 {

description "initial revision";

reference "Based on

TS 28.541 V15.X.XX";

}

grouping NWDAFFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based interface.";

type inet:domain-name;

}

leaf-list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//optional support

reference "TS 23.003";

type types3gpp:SNssai;

}

list managedNFProfile {

key idx;

min-elements 1;

uses types3gpp:ManagedNFProfile;

}

list commModelList {

min-elements 1;

key "groupId";

uses types5g3gpp:CommModel;

}

}

augment "/me3gpp:ManagedElement" {

list NWDAFFunction {

description "5G Core NWDAF Function";

reference "TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NWDAFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## H.5.18	module _3gpp-5gc-nrm-pcffunction@2020-08-06.yang

module _3gpp-5gc-nrm-pcffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-pcffunction;

prefix pcf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

description "This IOC represents the PCF function in 5GC. For more information about the PCF, see TS 23.501.";

reference "TS 28.541";

revision 2020-08-06 { reference "CR-0333"; }

revision 2020-08-06 {reference "CR-0331"; }

revision 2019-10-25 {reference "S5-194457 S5-193518"; }

revision 2019-05-22 {

description "initial revision";

}

grouping PCFFuntionGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based interface.";

type inet:domain-name;

}

leaf-list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//optional support

reference "TS 23.003";

type types3gpp:SNssai;

}

list managedNFProfile {

key idx;

min-elements 1;

uses types3gpp:ManagedNFProfile;

}

list commModelList {

min-elements 1;

key "groupId";

uses types5g3gpp:CommModel;

}

leaf dynamic5QISetRef {

type types3gpp:DistinguishedName;

description "DN of the Dynamic5QISet that the PCFFunction supports (is associated to).";

}

leaf configurable5QISetRef {

type types3gpp:DistinguishedName;

description "DN of the Configurable5QISet that the PCFFunction supports (is associated to).";

}

}

augment "/me3gpp:ManagedElement" {

list PCFFunction {

description "5G Core PCF Function";

reference "TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses PCFFuntionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## H.5.19	module _3gpp-5gc-nrm-seppfunction.yang

module _3gpp-5gc-nrm-seppfunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-seppfunction;

prefix sepp3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import ietf-inet-types { prefix inet; }

organization "3gpp SA5";

description "This IOC represents the SEPP function which support message filtering

and policing on inter-PLMN control plane interface. For more information about the SEPP, see TS 23.501.";

reference "TS 28.541";

revision 2020-08-03 { reference "CR-0321"; }

revision 2019-10-28 { reference S5-193518 ; }

typedef SEPPType {

reference "TS 23501";

type enumeration {

enum CSEPP {

value 0;

description "consumer SEPP";

}

enum PSEPP {

value 1;

description "producer SEPP";

}

}

}

grouping SEPPFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

container pLMNId {

description "PLMN Identifiers of the sepp.

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

uses types3gpp:PLMNId;

}

leaf sEPPType {

type sepp3gpp:SEPPType;

}

leaf sEPPId {

type uint16;

}

leaf fqdn {

description "The domain name of the SEPP.";

type inet:domain-name;

}

}

augment "/me3gpp:ManagedElement" {

list SEPPFunction {

description "5G Core SEPP Function";

reference "TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses SEPPFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;    }

}

}

## H.5.19a	module _3gpp-5gc-nrm- externalseppfunction@2019-11-17.yang

module _3gpp-5gc-nrm-externalseppfunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-extternalseppfunction;

prefix extsepp3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import ietf-inet-types { prefix inet; }

organization "3gpp SA5";

description "This IOC represents the external SEPP function which support message filtering

and policing on inter-PLMN control plane interface. For more information about the SEPP, see TS 23.501.";

reference "TS 28.541";

revision 2019-11-17 {

description "initial revision";

reference "Based on

TS 28.541 V16.X.XX";

}

grouping ExternalSEPPFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

container pLMNId {

description "PLMN Identifiers of the sepp.

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

uses types3gpp:PLMNId;

}

leaf sEPPId {

type uint16;

}

leaf fqdn {

description "The domain name of the SEPP.";

type inet:domain-name;

}

}

augment "/me3gpp:ManagedElement" {

list ExternalSEPPFunction {

description "5G Core SEPP Function";

reference "TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalSEPPFunctionGrp;

}

}

}

}

## H.5.20	module _3gpp-5gc-nrm-smffunction@2020-08-06.yang

module _3gpp-5gc-nrm-smffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-smffunction;

prefix smf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-top { prefix top3gpp; }

description "SMFFunction derived from basic ManagedFunction.";

revision 2020-08-06 { reference "CR-0333"; }

revision 2020-06-03 { reference "CR-0286"; }

revision 2019-10-25 { reference "S5-194457 S5-193518"; }

revision 2019-05-31 {

description "Ericsson refactoring.";

}

revision 2018-08-07 {

description "Initial revision";

}

grouping SMFFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

min-elements 1;

description "A list of PLMN identifiers (Mobile Country Code and Mobile Network Code).";

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf-list nRTACList {

description "List of Tracking Area Codes (legacy TAC or extended TAC)

where the represented management function is serving.";

reference "TS 38.413 clause 9.3.3.10";

min-elements 1;

config false;

type types3gpp:Tac;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based interface.";

type inet:domain-name;

}

leaf-list sNSSAIList {

min-elements 0;

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

reference "TS 23.003";

type types3gpp:SNssai;

}

list managedNFProfile {

key idx;

min-elements 1;

uses types3gpp:ManagedNFProfile;

}

list commModelList {

min-elements 1;

key "groupId";

uses types5g3gpp:CommModel;

}

leaf configurable5QISetRef {

type types3gpp:DistinguishedName;

description "DN of the Configurable5QISet that the SMFFunction supports (is associated to).";

}

leaf dynamic5QISetRef {

type types3gpp:DistinguishedName;

description "DN of the Dynamic5QISet that the SMFFunction supports (is associated to).";

}

}

augment "/me3gpp:ManagedElement" {

list SMFFunction {

description "5G Core SMF Function";

reference "TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses SMFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## H.5.21	module _3gpp-5gc-nrm-smsffunction@2019-10-25.yang

module _3gpp-5gc-nrm-smsffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-smsffunction;

prefix smsf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

description "This IOC represents the SMSF function defined in TS 23.501.";

reference "TS 28.541";

revision 2019-10-25 { reference "S5-194457 S5-195427 S5-193518"; }

revision 2019-05-15 {

description "initial revision";

}

grouping SMSFFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

list managedNFProfile {

key idx;

min-elements 1;

uses types3gpp:ManagedNFProfile;

}

list commModelList {

min-elements 1;

key "groupId";

uses types5g3gpp:CommModel;

}

}

augment "/me3gpp:ManagedElement" {

list SMSFFunction {

description "5G Core SMSF Function";

reference "TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses SMSFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## H.5.22	module _3gpp-5gc-nrm-udmfunction@2019-10-25.yang

module _3gpp-5gc-nrm-udmfunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-udmfunction;

prefix udm3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

description "This IOC represents the UDM function in 5GC. For more information about the UDM, see TS 23.501.";

reference "TS 28.541";

revision 2019-10-25 { reference "S5-194457 S5-195427 S5-193518"; }

revision 2019-05-22 {

description "initial revision";

}

grouping UDMFuntionGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based interface.";

type inet:domain-name;

}

leaf-list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//optional support

reference "TS 23.003";

type types3gpp:SNssai;

}

list managedNFProfile {

key idx;

min-elements 1;

uses types3gpp:ManagedNFProfile;

}

list commModelList {

min-elements 1;

key "groupId";

uses types5g3gpp:CommModel;

}

}

augment "/me3gpp:ManagedElement" {

list UDMFunction {

description "5G Core UDM Function";

reference "TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses UDMFuntionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## H.5.23	module _3gpp-5gc-nrm-udrfunction@2019-10-25.yang

module _3gpp-5gc-nrm-udrfunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-udrfunction;

prefix udr3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

description "This IOC represents the UDR function in 5GC. For more information about the UDR, see TS 23.501.";

reference "TS 28.541";

revision 2019-10-25 { reference "S5-194457 S5-195427 S5-193518"; }

revision 2019-05-22 {

description "initial revision";

}

grouping UDRFuntionGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based interface.";

type inet:domain-name;

}

leaf-list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//optional support

reference "TS 23.003";

type types3gpp:SNssai;

}

list managedNFProfile {

key idx;

min-elements 1;

uses types3gpp:ManagedNFProfile;

}

}

augment "/me3gpp:ManagedElement" {

list UDRFunction {

description "5G Core UDR Function";

reference "TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses UDRFuntionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## H.5.24	module _3gpp-5gc-nrm-udsffunction@2019-10-25.yang

module _3gpp-5gc-nrm-udsffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-udsffunction;

prefix udsf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

description "This IOC represents the UDSF function which can be interacted with any other 5GC NF defined in TS 23.501.";

reference "TS 28.541";

revision 2019-10-25 { reference "S5-194457 S5-195427 S5-193518"; }

revision 2019-05-22 {

description "initial revision";

}

grouping UDSFFuntionGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based interface.";

type inet:domain-name;

}

leaf-list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//optional support

reference "TS 23.003";

type types3gpp:SNssai;

}

list managedNFProfile {

key idx;

min-elements 1;

uses types3gpp:ManagedNFProfile;

}

}

augment "/me3gpp:ManagedElement" {

list UDSFFunction {

description "5G Core UDSF Function";

reference "TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses UDSFFuntionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;    }

}

}

## H.5.25	module _3gpp-5gc-nrm-upffunction@2019-10-25.yang

module _3gpp-5gc-nrm-upffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-upffunction;

prefix upf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-top { prefix top3gpp; }

description "UPFFunction derived from basic ManagedFunction.";

revision 2019-10-25 { reference "S5-194457 S5-193518"; }

revision 2019-05-31 {

description "Ericsson refactoring.";

}

revision 2018-08-07 {

description "Initial revision";

}

grouping UPFFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "A list of PLMN identifiers (Mobile Country Code and Mobile Network Code).";

min-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf-list nRTACList {

description "List of Tracking Area Codes (legacy TAC or extended TAC)

where the represented management function is serving.";

reference "TS 38.413 clause 9.3.3.10";

min-elements 1;

config false;

type types3gpp:Tac;

}

leaf-list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

min-elements 0;

reference "TS 23.003";

type types3gpp:SNssai;

}

list managedNFProfile {

key idx;

min-elements 1;

uses types3gpp:ManagedNFProfile;

}

list commModelList {

min-elements 1;

key "groupId";

uses types5g3gpp:CommModel;

}

}

augment /me3gpp:ManagedElement {

list UPFFunction {

description "5G Core UPF Function";

reference "TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses UPFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## H.5.26	module _3gpp-5gc-nrm-scpfunction.yang

module _3gpp-5gc-nrm-scpfunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-scpfunction;

prefix scp3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

description "This IOC represents the SCP function in 5GC. For more information about the SCP, see TS 23.501.";

reference "TS 28.541";

revision 2019-10-20 {

description "initial revision";

reference "Based on

TS 28.541 V16.X.XX";

}

grouping SCPFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

leaf address {

description "The host address of the SCP.";

type inet:host;

}

list supportedFuncList {

min-elements 1;

key "function";

uses types5g3gpp:SupportedFunc;

}

}

augment "/me3gpp:ManagedElement" {

list SCPFunction {

description "5G Core SCP Function";

reference "TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses SCPFunctionGrp;

}

}

}

}

## H.5.27	module _3gpp-5gc-nrm-neffunction.yang

module _3gpp-5gc-nrm-neffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-neffunction;

prefix nef3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

description "This IOC represents the NEF function in 5GC. For more information about the NEF, see TS 23.501.";

reference "TS 28.541";

revision 2019-10-20 {

description "initial revision";

reference "Based on

TS 28.541 V16.X.XX";

}

grouping NEFFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based interface.";

type inet:domain-name;

}

leaf-list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

type types3gpp:SNssai;

}

leaf-list capabilityList {

description "List of supported capabilities of the NEF.";

reference "TS 23.003";

type string;

}

leaf isINEF {

type boolean;

}

leaf isCAPIFSup {

type boolean;

}

}

augment "/me3gpp:ManagedElement" {

list NEFFunction {

description "5G Core NEF Function";

reference "TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NEFFunctionGrp;

}

}

}

}

## H.5.28	module _3gpp-5gc-nrm-QFQoSMonitoringControl.yang

module _3gpp-5gc-nrm-QFQoSMonitoringControl {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-QFQoSMonitoringControl;

prefix qFQMCtrl3gpp;

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5gc-nrm-smffunction { prefix smf3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";  contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the capabilities and properties for control

of QoS monitoring per QoS flow per UE for URLLC service defined

in TS 23.501.";

reference "TS 28.541";

revision 2020-08-03 { reference "CR-0321"; }

revision 2020-04-10 { reference "S5-202101";   }

grouping QFPacketDelayThresholdsTypeGrp {

leaf thresholdDl {

type uint32;

units milliseconds;

description "Downlink threshold";

}

leaf thresholdUl {

type uint32;

units milliseconds;

description "Uplink threshold";

}

leaf thresholdRtt {

type uint32;

units milliseconds;

description "Round trip threshold";

}

}

grouping QFQoSMonitoringControlGrp {

description "Represents the QFQoSMonitoringControl IOC.";

reference "TS 28.541";

leaf qFQoSMonitoringState {

description "The state of QoS monitoring per QoS flow per UE.";

mandatory true;

type enumeration {

enum ENABLED;

enum DISABLED;

}

}

leaf-list qFMonitoredSNSSAIs {

description "The S-NSSAIs for which the QoS monitoring per QoS flow

per UE is to be performed.";

reference "TS 23.003";

type types3gpp:SNssai;

}

leaf-list qFMonitored5QIs {

description "The 5QIs for which the QoS monitoring per QoS flow

per UE is to be performed.";

reference "TS 23.501";

type uint32 {

range "0..255";

}

}

leaf isEventTriggeredQFMonitoringSupported {

description "It indicates whether the event based QoS monitoring

reporting per QoS flow per UE is supported.";

mandatory true;

reference "TS 29.244";

type boolean;

}

leaf isPeriodicQFMonitoringSupported {

description "It indicates whether the periodic QoS monitoring reporting

per QoS flow per UE is supported.";

mandatory true;

reference "TS 29.244";

type boolean;

}

leaf isSessionReleasedQFMonitoringSupported {

description "It indicates whether the session release based QoS monitoring

reporting  per QoS flow per UE is supported.";

mandatory true;

reference "TS 29.244";

type boolean;

}

list qFPacketDelayThresholds {

key "idx";

min-elements 1;

max-elements 1;

description "It specifies the thresholds for reporting the packet delay

between PSA and UE for QoS monitoring per QoS flow per UE.";

leaf idx { type uint32 ; }

uses QFPacketDelayThresholdsTypeGrp;

}

leaf qFMinimumWaitTime {

description "It specifies the minimum waiting time (in seconds) between

two consecutive reports for event triggered QoS monitoring reporting

per QoS flow per UE.";

type uint32;

}

leaf qFMeasurementPeriod {

description "It specifies the period (in seconds) for reporting the packet delay for QoS monitoring per QoS flow per UE.";

type uint32;

}

}

augment "/me3gpp:ManagedElement/smf3gpp:SMFFunction" {

list QFQoSMonitoringControl {

description "Represents the QFQoSMonitoringControl IOC.";

reference "TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses QFQoSMonitoringControlGrp;

}

}

}

}

## H.5.29	module _3gpp-5gc-nrm-GtpUPathQoSMonitoringControl.yang

module _3gpp-5gc-nrm-GtpUPathQoSMonitoringControl {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-GtpUPathQoSMonitoringControl;

prefix gtpUPathQMCtrl3gpp;

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5gc-nrm-smffunction { prefix smf3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the capabilities and properties for control

of GTP-U path QoS monitoring defined in TS 23.501.";

reference "TS 28.541";

revision 2020-08-03 { reference "CR-0321"; }

revision 2020-04-10 { reference "S5-202103"; }

grouping GtpUPathDelayThresholdsType {

leaf n3AveragePacketDelayThreshold {

mandatory true;

type uint32;

}

leaf n3MinPacketDelayThreshold {

mandatory true;

type uint32;

}

leaf n3MaxPacketDelayThreshold {

mandatory true;

type uint32;

}

leaf n9AveragePacketDelayThreshold {

mandatory true;

type uint32;

}

leaf n9MinPacketDelayThreshold {

mandatory true;

type uint32;

}

leaf n9MaxPacketDelayThreshold {

mandatory true;

type uint32;

}

}

grouping GtpUPathQoSMonitoringControlGrp {

description "Represents the GtpUPathQoSMonitoringControl IOC.";

leaf gtpUPathQoSMonitoringState {

description "The state of GTP-U path QoS monitoring.";

mandatory true;

type enumeration {

enum ENABLED;

enum DISABLED;

}

}

leaf-list gtpUPathMonitoredSNSSAIs {

description "The S-NSSAIs for which the the GTP-U path QoS monitoring is

to be performed.";

reference "TS 23.003";

type types3gpp:SNssai;

}

leaf-list monitoredDSCPs {

description "The DSCPs for which the GTP-U path QoS monitoring is to be

performed.";

reference "TS 29.244";

type uint32;

}

leaf isEventTriggeredGtpUPathMonitoringSupported {

description "It indicates whether the event triggered GTP-U path QoS

monitoring reporting based on thresholds is supported.";

mandatory true;

reference "TS 29.244";

type boolean;

}

leaf isPeriodicGtpUMonitoringSupported {

description "It indicates whether the periodic GTP-U path QoS monitoring

reporting is supported.";

mandatory true;

reference "TS 29.244";

type boolean;

}

leaf isImmediateGtpUMonitoringSupported {

description "It indicates whether the immediate GTP-U path QoS monitoring

reporting is supported.";

mandatory true;

reference "TS 29.244";

type boolean;

}

list gtpUPathDelayThresholds {

key n3AveragePacketDelayThreshold;

// if max-elements is increased later, the key may need to be modified

min-elements 1;

max-elements 1;

description "It specifies the thresholds for reporting the packet delay

for the GTO-U path QoS monitoring.";

uses GtpUPathDelayThresholdsType;

}

leaf gtpUPathMinimumWaitTime {

description "It specifies the minimum waiting time (in seconds) between

two consecutive reports for event triggered GTP-U path QoS monitoring

reporting.";

type uint32;

}

leaf gtpUPathMeasurementPeriod {

description "It specifies the period (in seconds) for reporting the packet

delay for GTP-U path QoS monitoring.";

type uint32;

}

}

augment "/me3gpp:ManagedElement/smf3gpp:SMFFunction " {

list GtpUPathQoSMonitoringControl {

description "Specifies the capabilities and properties for control of

GTP-U path QoS monitoring. For more information about the GTP-U path

QoS monitoring.";

reference "TS 23.501";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses GtpUPathQoSMonitoringControlGrp;

}

}

}

}

## H.5.30	module _3gpp-5gc-nrm-Configurable5QISet.yang

module _3gpp-5gc-nrm-configurable5qiset {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-configurable5qiset;

prefix Conf5QIs3gpp;

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the non-standardized 5QIs, including

their QoS characteristics, that need to be pre-configured

(and configurable) to the 5G NFs.";

reference "TS 28.541";

revision 2020-08-03 { reference "CR-0321"; }

revision 2020-06-03 { reference "CR-0286"; }

grouping PacketErrorRate {

leaf scalar {

type uint32 {

range 0..9 ;

}

mandatory true;

description "The Packet Error Rate of a 5QI expressed as Scalar x 10-k

where k is the Exponent.";

}

leaf exponent {

type uint32 {

range 0..9 ;

}

mandatory true;

description "The Packet Error Rate of a 5QI expressed as Scalar x 10-k,

where k is the Exponent.";

}

}

grouping FiveQICharacteristics {

leaf fiveQIValue {

type uint32 {

range 0..255 ;

}

mandatory true;

description "Identifies the 5QI value.";

}

leaf resourceType {

type enumeration {

enum GBR;

enum NON_GBR;

}

mandatory true;

description "It indicates the Resource Type of a 5QI, as specified

in TS 23.501 ";

}

leaf priorityLevel {

type uint32 {

range 0..127 ;

}

}

leaf packetDelayBudget {

type uint32 {

range 0..1023 ;

}

description "Indicates the Packet Delay Budget (in unit of 0.5ms)of a 5QI,

as specified in TS 23.501 ";

}

list packetErrorRate {

key "scalar exponent";

min-elements 0;

max-elements 1;

uses PacketErrorRate;

reference "TS 23.501";

}

leaf averagingWindow {

type uint32 {

range 0..4095 ;

}

units ms;

reference "TS 23.501";

}

leaf maximumDataBurstVolume {

type uint32{

range 0..4095 ;

}

units byte;

}

}

grouping Configurable5QISetGrp {

description "Represents the Configurable5QISet IOC.";

list configurable5QIs {

key "fiveQIValue";

uses FiveQICharacteristics;

}

}

grouping Configurable5QISetSubtree {

list Configurable5QISet {

description "Specifies the non-standardized 5QIs, including their QoS

characteristics, that need to be pre-configured (and configurable) to

the 5G NFs, see TS 23.501.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses Configurable5QISetGrp;

}

}

}

augment "/subnet3gpp:SubNetwork" {

uses Configurable5QISetSubtree;

}

augment "/me3gpp:ManagedElement" {

uses Configurable5QISetSubtree;

}

}

## H.5.31	module _3gpp-5gc-nrm-FiveQiDscpMappingSet.yang

module _3gpp-5gc-nrm-FiveQiDscpMappingSet {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-FiveQiDscpMappingSet;

prefix FiveQiDscpMapping3gpp;

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-5gc-nrm-smffunction { prefix smf3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description " This IOC represents the set of mapping between 5QIs and DSCP.";

reference "TS 28.541";

revision 2020-08-03 { reference "CR-0321"; }

revision 2020-05-27 { reference "CR-0287"; }

grouping FiveQiDscpMapping {

leaf-list fiveQIValues {

type uint32 {

range 0..255 ;

}

min-elements 1;

description " Identifies the 5QI values that are mapped to a same DSCP, as specified in TS 28.541.";

}

leaf dscp {

type uint32 {

range 0..255 ;

}

mandatory true;

}

}

grouping FiveQiDscpMappingSetGrp {

description "Represents the FiveQiDscpMappingSet IOC.";

list FiveQiDscpMappingList {

key "dscp";

uses FiveQiDscpMapping;

}

}

grouping FiveQiDscpMappingSetSubtree {

list FiveQiDscpMappingSet {

description "Specifies the mapping between 5QIs and DSCPs.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses FiveQiDscpMappingSetGrp;

}

}

}

augment "/me3gpp:ManagedElement/smf3gpp:SMFFunction" {

uses FiveQiDscpMappingSetSubtree;

}

}

## H.5.32	module _3gpp-5gc-nrm-PredefinedPccRuleSet.yang

module _3gpp-5gc-nrm-predefinedpccruleset {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-predefinedpccruleset;

prefix PredPccRules3gpp;

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-5gc-nrm-smffunction { prefix smf3gpp; }

import _3gpp-5gc-nrm-pcffunction { prefix pcf3gpp; }

import _ietf-yang-types { prefix yang; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the predefined PCC rules, which are configured to SMF and referenced by PCF.";

reference "TS 28.541";

revision 2020-08-21 { reference "CR-0330"; }

grouping TscaiInputContainer {

description "It specifies the transports TSCAI input parameters for TSC traffic at the ingress interface of the DS-TT/UE for a PCC rule, see TS 29.512 [60]";

leaf periodicity {

type uint32;

mandatory false;

description "It identifies the time period between the start of two bursts in reference to the TSN GM, see TS 29.571 [61].";

}

leaf burstArrivalTime {

type yang:date-and-time;

mandatory false;

description "It Indicates the arrival time (in date-time format) of the data burst in reference to the TSN GM, see TS 29.571 [61].";

}

}

grouping ConditionData {

description "It specifies the specifies the condition data for a PCC rule.";

leaf condId {

type string;

mandatory true;

description "It uniquely identifies the condition data.";

}

leaf activationTime {

type yang:date-and-time;

mandatory false;

description " It indicates the time (in date-time format) when the decision data shall be activated, see TS 29.512 [60] and TS 29.571 [61].";

}

leaf deactivationTime {

type yang:date-and-time;

mandatory false;

description "It indicates the time (in date-time format) when the decision data shall be deactivated, see TS 29.512 [60] and TS 29.571 [81].";

}

leaf accessType {

type enumeration {

enum 3GPP_ACCESS;

enum NON_3GPP_ACCESS;

}

mandatory false;

description "It provides the condition of access type of the UE when the session AMBR shall be enforced, see TS 29.512 [60].";

}

leaf ratType {

type enumeration {

enum NR;

enum EUTRA;

enum WLAN;

enum VIRTUAL;

enum NBIOT;

enum WIRELINE;

enum WIRELINE_CABLE;

enum WIRELINE_BBF;

enum LTE-M;

enum NR_U;

enum EUTRA_U;

enum TRUSTED_N3GA;

enum TRUSTED_WLAN;

enum UTRA;

enum GERA;

}

mandatory false;

description "It provides the condition of RAT type of the UE when the session AMBR shall be enforced, see TS 29.512 [60] and TS 29.571 [61].";

}

}

grouping SteeringMode {

description "It specifies the traffic distribution rule, see TS 29.512 [60].";

leaf steerModeValue {

type enumeration {

enum ACTIVE_STANDBY;

enum LOAD_BALANCING;

enum SMALLEST_DELAY;

enum PRIORITY_BASED;

}

mandatory true;

description "It indicates the value of the steering mode, see TS 29.512 [60].";

}

leaf active {

type enumeration {

enum 3GPP_ACCESS;

enum NON_3GPP_ACCESS;

}

mandatory false;

description "It indicates the active access, see TS 29.571 [61].";

}

leaf standby {

type enumeration {

enum 3GPP_ACCESS;

enum NON_3GPP_ACCESS;

}

mandatory false;

description "It indicates the Standby access, see TS 29.571 [61].";

}

leaf threeGLoad {

type uint8 {

range 0..100;

}

mandatory false;

description "It indicates the traffic load to steer to the 3GPP Access expressed in one percent.";

}

leaf prioAcc {

type enumeration {

enum 3GPP_ACCESS;

enum NON_3GPP_ACCESS;

}

mandatory false;

description "It indicates the high priority access, see TS 29.571 [61].";

}

}

grouping UpPathChgEvent {

description "It specifies the information about the AF subscriptions of the UP path change, see TS 29.512 [60]";

leaf notificationUri {

type string;

mandatory true;

description "It provides notification address (Uri) of AF receiving the event notification.";

}

leaf notifCorreId {

type string;

mandatory true;

description "It is used to set the value of Notification Correlation ID in the notification sent by the SMF, see TS 29.512 [60].";

}

leaf dnaiChgType {

type enumeration {

enum EARLY;

enum EARLY_LATE;

enum LATE;

}

mandatory true;

description "It indicates the type of DNAI change, see TS 29.512 [60].";

}

leaf afAckInd {

type boolean;

default false;

mandatory false;

description "It identifies whether the AF acknowledgement of UP path event notification is expected.";

}

}

grouping RouteInformation {

description "It specifies the traffic routing information.";

leaf ipv4Addr {

type string;

mandatory false;

description "It defines the Ipv4 address of the tunnel end point in the data network, formatted in the dotted decimal notation.";

}

leaf ipv6Addr {

type string;

mandatory false;

description "It defines the Ipv6 address of the tunnel end point in the data network.";

}

leaf portNumber {

type uint32;

mandatory true;

description " It defines the UDP port number of the tunnel end point in the data network, see TS 29.571 [61].";

}

}

grouping RouteToLocation {

description "It specifies a list of location which the traffic shall be routed to for the AF request.";

leaf dnai {

type string;

mandatory true;

description "It represents the DNAI (Data network access identifier), see TS 23.501 [2].";

}

container routeInfo{

description "It provides the traffic routing information.";

uses RouteInformation;

}

leaf routeProfId {

type string;

mandatory false;

description "It identifies the routing profile.";

}

}

grouping RedirectInformaton {

description "It specifies the redirect information for traffic control in the PCC rule.";

leaf redirectEnabled {

type boolean;

mandatory true;

description "It indicates whether the redirect instruction is enabled.";

}

leaf redirectAddressType {

type enumeration {

enum IPV4_ADDR;

enum IPV6_ADDR;

enum URL;

enum SIP_URI;

}

mandatory true;

description "It indicates the type of redirect address, see TS 29.512 [60].";

}

leaf redirectServerAddress {

type string;

mandatory true;

description "It indicates the address of the redirect server.";

}

}

grouping TrafficControlDataInformation {

description "It specifies the traffic control data for a service flow of a PCC rule.";

leaf tcId {

type string;

mandatory true;

description "It univocally identifies the traffic control policy data within a PDU session.";

}

leaf flowStatus {

type enumeration {

enum ENABLED-UPLINK;

enum ENABLED-DOWNLINK;

enum ENABLED;

enum DISABLED;

enum REMOVED;

}

mandatory true;

description "It represents whether the service data flow(s) are enabled or disabled.";

}

container redirectInfo {

description "It contains the redirect information indicating whether the detected application traffic should be redirected to another controlled address.";

uses RedirectInformaton;

}

container addRedirectInfo {

description "It contains the additional redirect information indicating whether the detected application traffic should be redirected to another controlled address.";

list redirectInfo {

description "The list of redirect information indicating whether the detected application traffic should be redirected to another controlled address.";

key "redirectServerAddress";

uses RedirectInformaton;

}

}

leaf muteNotif {

type boolean;

mandatory false;

default false;

description "It indicates whether applicat'on's start or stop notification is to be muted.";

}

leaf trafficSteeringPolIdDl {

type string;

mandatory false;

description "It references to a pre-configured traffic steering policy for downlink traffic at the SMF, see TS 29.512 [60].";

}

leaf trafficSteeringPolIdUl {

type string;

mandatory false;

description "It references to a pre-configured traffic steering policy for uplink traffic at the SMF, see TS 29.512 [60].";

}

container routeToLocs {

description "It provides a list of location which the traffic shall be routed to for the AF request.";

list routeToLoc {

description "The list of location which the traffic shall be routed to for the AF request.";

key "dnai";

uses RouteToLocation;

}

}

uses UpPathChgEvent;

leaf steerFun {

type enumeration {

enum MPTCP;

enum ATSSS_LL;

}

mandatory false;

description "It indicates the applicable traffic steering functionality, see TS 29.512 [60].";

}

container steerModeDl {

description "It provides the traffic distribution rule across 3GPP and Non-3GPP accesses to apply for downlink traffic.";

uses SteeringMode;

}

container steerModeUl {

description "It provides the traffic distribution rule across 3GPP and Non-3GPP accesses to apply for uplink traffic.";

uses SteeringMode;

}

leaf mulAccCtrl {

type enumeration {

enum ALLOWED;

enum NOT_ALLOWED;

}

mandatory false;

description "It indicates whether the service data flow, corresponding to the service data flow template, is allowed or not allowed.";

}

}

grouping ARP {

description "It specifies the allocation and retention priority of a QoS control policy.";

leaf priorityLevel {

type uint8 {

range 1..15;

}

mandatory true;

description "It defines the relative importance of a resource request.";

}

leaf preemptCap {

type enumeration {

enum NOT_PREEMPT;

enum MAY_PREEMPT;

}

mandatory true;

description "It defines whether a service data flow may get resources that were already assigned to another service data flow with a lower priority level.";

}

leaf preemptVuln {

type enumeration {

enum NOT_PREEMPTABLE;

enum PREEMPTABLE;

}

mandatory true;

description "It defines whether a service data flow may lose the resources assigned to it in order to admit a service data flow with higher priority level.";

}

}

grouping QosDataInformation {

description "It specifies the QoS control policy data for a service flow of a PCC rule.";

leaf qosId {

type string;

mandatory true;

description "It identifies the QoS control policy data for a PCC rule.";

}

leaf fiveQIValue {

type uint8 {

range 0..255;

}

mandatory true;

description "It indicates the 5QI value.";

}

leaf maxbrUl {

type string;

mandatory false;

description "It represents the maximum uplink bandwidth.";

}

leaf maxbrDl {

type string;

mandatory false;

description "It represents the maximum downlink bandwidth.";

}

leaf gbrUl {

type string;

mandatory false;

description "It represents the guaranteed uplink bandwidth.";

}

leaf gbrDl {

type string;

mandatory false;

description "It represents the guaranteed downlink bandwidth.";

}

uses ARP;

leaf qosNotificationControl {

type boolean;

default false;

mandatory false;

description "It indicates whether notifications are requested from 3GPP NG-RAN when the GFBR can no longer (or again) be guaranteed for a QoS Flow during the lifetime of the QoS Flow.";

}

leaf reflectiveQos {

type boolean;

default false;

mandatory false;

description "Indicates whether the QoS information is reflective for the corresponding non-GBR service data flow";

}

leaf sharingKeyDl {

type string;

mandatory false;

description "It indicates, by containing the same value, what PCC rules may share resource in downlink direction.";

}

leaf sharingKeyUl {

type string;

mandatory false;

description "It indicates, by containing the same value, what PCC rules may share resource in uplink direction.";

}

leaf maxPacketLossRateDl {

type uint16 {

range 0..1000;

}

mandatory false;

description "It indicates the downlink maximum rate for lost packets that can be tolerated for the service data flow.";

}

leaf maxPacketLossRateUl {

type uint16 {

range 0..1000;

}

mandatory false;

description "It indicates the uplink maximum rate for lost packets that can be tolerated for the service data flow.";

}

leaf extMaxDataBurstVol {

type uint32 {

range 4096..2000000;

}

mandatory false;

description "It denotes the largest amount of data that is required to be transferred within a period of 5G-AN PDB, see TS 29.512 [60].";

}

}

grouping EthFlowDescription {

description "It describes an Ethernet flow.";

leaf destMacAddr {

type string;

mandatory true;

description "It specifies the destination MAC address formatted in the hexadecimal notation according to clause 1.1 and clause 2.1 of IETF RFC 7042 [63].";

}

leaf ethType {

type string;

mandatory true;

description "A two-octet string that represents the Ethertype, as described in IEEE 802.3 [64] and IETF RFC 7042 [63] in hexadecimal representation.";

}

leaf fDesc {

type string;

mandatory false;

description "It contains the flow description for the Uplink or Downlink IP flow. It shall be present when the ethtype is IP.";

}

leaf fDir {

type enumeration {

enum DOWNLINK;

enum UPLINK;

}

mandatory true;

description "It indicates the packet filter direction.";

}

leaf sourceMacAddr {

type string;

mandatory true;

description "It specifies the source MAC address formatted in the hexadecimal notation according to clause 1.1 and clause 2.1 of IETF RFC 7042 [63].";

}

leaf-list vlanTags {

type string;

description "It specifies the Customer-VLAN and/or Service-VLAN tags containing the VID, PCP/DEI fields as defined in IEEE 802.1Q [65] and IETF RFC 7042 [63]. The first/lower instance in the array stands for the Customer-VLAN tag and the second/higher instance in the array stands for the Service-VLAN tag.";

}

leaf srcMacAddrEnd {

type string;

mandatory false;

description "It specifies the source MAC address end. If this attribute is present, the sourceMacAddr attribute specifies the source MAC address start. E.g. srcMacAddrEnd with value 00-10-A4-23-3E-FE and sourceMacAddr with value 00-10-A4-23-3E-02 means all MAC addresses from 00-10-A4-23-3E-02 up to and including 00-10-A4-23-3E-FE.";

}

leaf destMacAddrEnd {

type string;

mandatory false;

description "It specifies the destination MAC address end. If this attribute is present, the destMacAddr attribute specifies the destination MAC address start.";

}

}

grouping FlowInformation {

description "It specifies the flow information of a PCC rule.";

leaf flowDescription {

type string;

mandatory true;

description "It defines a packet filter for an IP flow.";

}

uses EthFlowDescription;

leaf packFiltId {

type string;

mandatory true;

description "It is the identifier of the packet filter.";

}

leaf packetFilterUsage {

type boolean;

default false;

description "It indicates if the packet shall be sent to the UE.";

}

leaf tosTrafficClass {

type string;

mandatory true;

description "It contains the Ipv4 Type-of-Service and mask field or the Ipv6 Traffic-Class field and mask field.";

}

leaf spi {

type string;

mandatory true;

description "It is the security parameter index of the IPSec packet, see IETF RFC 4301 [66]";

}

leaf flowLabel {

type string;

mandatory false;

description "It specifies the Ipv6 flow label header field.";

}

leaf flowDirection {

type enumeration {

enum DOWNLINK;

enum UPLINK;

enum BIDIRECTIONAL;

enum UNSPECIFIED;

}

mandatory true;

description "It indicates the direction/directions that a filter is applicable.";

}

}

grouping PccRule {

description "It specifies the PCC rule, see TS 29.512 [60].";

leaf pccRuleId {

type string;

mandatory true;

description "It identifies the PCC rule.";

}

container flowInfoList {

description "It is a list of IP flow packet filter information.";

list flowInfo {

description "The list of IP flow packet filter information.";

key "packFiltId";

uses FlowInformation;

}

}

leaf applicationId {

type string;

default false;

mandatory false;

description "A reference to the application detection filter configured at the UPF.";

}

leaf appDescriptor {

type string;

mandatory false;

description "It is the ATSSS rule application descriptor.";

}

leaf contentVersion {

type uint8;

mandatory false;

description "Indicates the content version of the PCC rule.";

}

leaf precedence {

type uint8 {

range 0..255;

}

mandatory false;

description "It indicates the order in which this PCC rule is applied relative to other PCC rules within the same PDU session.";

}

leaf afSigProtocol {

type enumeration {

enum NO_INFORMATION;

enum SIP;

}

mandatory false;

description "Indicates the protocol used for signalling between the UE and the AF, the default value is NO_INFORMATION.";

}

leaf isAppRelocatable {

type boolean;

default false;

mandatory false;

description "It indicates the application relocation possibility, the default value is NO_INFORMATION.";

}

leaf isUeAddrPreserved {

type boolean;

default false;

mandatory false;

description "It Indicates whether UE IP address should be preserved.";

}

container qosData {

description "It contains the QoS control policy data for a PCC rule.";

list qosDataInfo {

description "The list of QoS control policy data.";

key "qosId";

uses QosDataInformation;

}

}

container altQosParams {

description "It contains the QoS control policy data for the Alternative QoS parameter sets of the service data flow.";

list qosDataInfo {

description "The list of QoS control policy data.";

key "qosId";

uses QosDataInformation;

}

}

container trafficControlData {

description "It contains the traffic control policy data for a PCC rule.";

list trafficControlDataInfo {

description "The list of traffic control policy data.";

key "tcId";

uses TrafficControlDataInformation;

}

}

uses ConditionData;

container tscaiInputUl {

description "It contains transports TSCAI input parameters for TSC traffic at the ingress interface of the DS-TT/UE (uplink flow direction).";

uses TscaiInputContainer;

}

container tscaiInputDl {

description "It contains transports TSCAI input parameters for TSC traffic at the ingress of the NW-TT (downlink flow direction).";

uses TscaiInputContainer;

}

}

grouping PredefinedPccRuleSetGrp {

description "Represents the PredefinedPccRuleSet IOC.";

list PredefinedPccRules {

description "The list of predefined PCC rules.";

key "pccRuleId";

uses PccRule;

}

}

grouping PredefinedPccRuleSetSubtree {

description "It specifies the PredefinedPccRuleSet IOC with inherited attributes.";

list PredefinedPccRuleSet {

description "Specifies the predefined PCC rules.";

key "id";

uses top3gpp:Top_Grp;

container attributes {

description "It contains the attributes defined specifically in the PredefinedPccRuleSet IOC.";

uses PredefinedPccRuleSetGrp;

}

}

}

augment "/me3gpp:ManagedElement/smf3gpp:SMFFunction" {

description "It specifies the containment relation of PredefinedPccRuleSet MOI with SMFFunction MOI.";

uses PredefinedPccRuleSetSubtree;

}

augment "/me3gpp:ManagedElement/pcf3gpp:PCFFunction" {

description "It specifies the containment relation of PredefinedPccRuleSet MOI with PCFFunction MOI.";

uses PredefinedPccRuleSetSubtree;

}

}

## H.5.33	module _3gpp-5gc-nrm-dynamic5QISet@2020-08-06.yang

module _3gpp-5gc-nrm-dynamic5qiset {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-dynamic5qiset;

prefix dyn5QIs3gpp;

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the dynamic 5QIs including their QoS characteristics.";

reference "TS 28.541";

revision 2020-08-06 { reference "CR-0333"; }

grouping Dynamic5QISetGrp {

description "Represents the Dynamic5QISet IOC.";

list dynamic5QIs {

key "fiveQIValue";

uses Conf5QIs3gpp:FiveQICharacteristics;

}

}

grouping Dynamic5QISetSubtree {

list Dynamic5QISet {

description "Specifies the dynamic 5QIs including their QoS

characteristics, see TS 23.501.";

key "id";

uses top3gpp:Top_Grp;

container attributes {

uses Dynamic5QISetGrp;

}

}

}

augment "/subnet3gpp:SubNetwork" {

uses Dynamic5QISetSubtree;

}

augment "/me3gpp:ManagedElement" {

uses Dynamic5QISetSubtree;

}

}

# H.6	Void

# H.7	Mount information

_3gpp-5gc-nrm-affunction.yang

_3gpp-5gc-nrm-amffunction.yang

_3gpp-5gc-nrm-amfregion.yang

_3gpp-5gc-nrm-amfset.yang

_3gpp-5gc-nrm-ausffunction.yang

_3gpp-5gc-nrm-dnfunction.yang

_3gpp-5gc-nrm-ep.yang

_3gpp-5gc-nrm-externalnrffunction.yang

_3gpp-5gc-nrm-externalnssffunction.yang

_3gpp-5gc-nrm-lmffunction.yang

_3gpp-5gc-nrm-n3iwffunction.yang

_3gpp-5gc-nrm-nfprofile.yang

_3gpp-5gc-nrm-nfservice.yang

_3gpp-5gc-nrm-ngeirfunction.yang

_3gpp-5gc-nrm-nrffunction.yang

_3gpp-5gc-nrm-nssffunction.yang

_3gpp-5gc-nrm-nwdaffunction.yang

_3gpp-5gc-nrm-pcffunction.yang

_3gpp-5gc-nrm-seppfunction.yang

_3gpp-5gc-nrm-smffunction.yang

_3gpp-5gc-nrm-smsffunction.yang

_3gpp-5gc-nrm-udmfunction.yang

_3gpp-5gc-nrm-udrfunction.yang

_3gpp-5gc-nrm-udsffunction.yang

_3gpp-5gc-nrm-upffunction.yang

If the above files are mounted the yang files described in clause E.7 shall also be mounted.

Annex I (normative):
XML definitions for network slice

# I.1	General

This annex contains the XML definitions for the network slice NRM, in accordance with network slice NRM Information Model definitions specified in clause 6.

# I.2	Architectural features

The overall architectural feature of network slice information model is specified in clause 6, this clause specifies features that are specific to the Schema definitions.

The XML definitions of the present document specify the schema for a configuration content, which can be included in a configuration file for Bulk configuration management operations.

# I.3	Mapping

## I.3.1	General mapping

An IOC maps to an XML element of the same name as the IOC's name in the Information Model. An IOC attribute maps to a sub-element of the corresponding IOC's XML element, and the name of this sub-element is the same as the attribute's name in the Information Model.

## I.3.2	Information Object Class (IOC) mapping

The mapping is not present in the current version of the present document.

# I.4	Solution Set (SS) definitions

## I.4.1	XML definition structure

The overall description of the file format of configuration data XML files is provided by TS 32.616 [33].

This annex defines the NRM-specific XML schema sliceNrm.xsd for the network slice Information Model defined in clause 6.

XML schema sliceNrm.xsd explicitly declares NRM-specific XML element types for the related NRM.

The definition of those NRM-specific XML element types complies with the generic mapping rules defined in TS 32.616 [33].

## I.4.2	Graphical representation

The graphical representation is not present in the current version of the present document.

## I.4.3	XML schema "sliceNrm.xsd"

<?xml version="1.0" encoding="UTF-8"?>

<!--

TS 28.541 network slice Network Resource Model

XML schema definition

sliceNrm.xsd

-->

<schema xmlns="http://www.w3.org/2001/XMLSchema"

xmlns:xn="http://www.3gpp.org/ftp/specs/archive/28_series/28.623#genericNrm"

xmlns:sl="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#sliceNrm"

xmlns:nn="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#nrNrm"

xmlns:ngc="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#ngcNrm"

xmlns:en="http://www.3gpp.org/ftp/specs/archive/28_series/28.659#eutranNrm"

xmlns:sm="http://www.3gpp.org/ftp/specs/archive/28_series/28.626#stateManagementIRP"

targetNamespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#sliceNrm" elementFormDefault="qualified">

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.623#genericNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#nrNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.541#ngcNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.659#eutranNrm"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.626#stateManagementIRP"/>

<simpleType name="MobilityLevel">

<restriction base="string">

<enumeration value="STATIONARY"/>

<enumeration value="NOMADIC"/>

<enumeration value="RESTRICTED MOBILITY"/>

<enumeration value="FULLY MOBILITY"/>

</restriction>

</simpleType>

<simpleType name="SharingLevel">

<restriction base="string">

<enumeration value="SHARED"/>

<enumeration value="NON-SHARED"/>

</restriction>

</simpleType>

<simpleType name="Category">

<restriction base="string">

<enumeration value="character"/>

<enumeration value="scalability"/>

</restriction>

</simpleType>

<simpleType name="Tagging">

<restriction base="string">

<enumeration value="performance"/>

<enumeration value="function"/>

<enumeration value="operation"/>

</restriction>

</simpleType>

<simpleType name="Exposure">

<restriction base="string">

<enumeration value="API"/>

<enumeration value="KPI"/>

</restriction>

</simpleType>

<complexType name="ServAttrCom">

<sequence>

<element name="category" type="sl:Category"/>

<element name="tagging" type="sl:Tagging" minOccurs="0"/>

<element name="exposure" type="sl:Exposure" minOccurs="0"/>

</sequence>

</complexType >

<simpleType name="DelayToleranceSupport">

<restriction base="string">

<enumeration value="NOT SUPPORTED"/>

<enumeration value="SUPPORTED"/>

</restriction>

</simpleType>

<simpleType name="DeterminCommAvailability">

<restriction base="string">

<enumeration value="NOT SUPPORTED"/>

<enumeration value="SUPPORTED"/>

</restriction>

</simpleType>

<simpleType name="UserMgmtOpenSupport">

<restriction base="string">

<enumeration value="NOT SUPPORTED"/>

<enumeration value="SUPPORTED"/>

</restriction>

</simpleType>

<simpleType name="V2XCommModelsV2XMode">

<restriction base="string">

<enumeration value="NOT SUPPORTED"/>

<enumeration value="SUPPORTED BY NR"/>

</restriction>

</simpleType>

<complexType name="DelayTolerance">

<sequence>

<element name="servAttrCom" type="sl:ServAttrCom"/>

<element name="support" type="sl:DelayToleranceSupport"/>

</sequence>

</complexType>

<complexType name="DeterminComm">

<sequence>

<element name="servAttrCom" type="sl:ServAttrCom"/>

<element name="availability" type="sl:DeterminCommAvailability"/>

<element name="periodicityList" type="string"/>

</sequence>

</complexType>

<complexType name="DLThpt">

<sequence>

<element name="servAttrCom" type="sl:ServAttrCom"/>

<element name="guaThpt" type="float"/>

<element name="maxThpt" type="float"/>

</sequence>

</complexType>

<complexType name="ULThpt">

<sequence>

<element name="servAttrCom" type="sl:ServAttrCom"/>

<element name="guaThpt" type="float" minOccurs="0"/>

<element name="maxThpt" type="float" minOccurs="0"/>

</sequence>

</complexType>

<complexType name="MaxPktSize">

<sequence>

<element name="servAttrCom" type="sl:ServAttrCom"/>

<element name="maxsize" type="integer"/>

</sequence>

</complexType>

<complexType name="KPIMonitoring">

<sequence>

<element name="servAttrCom" type="sl:ServAttrCom"/>

<element name="kPIList" type="string"/>

</sequence>

</complexType>

<complexType name="UserMgmtOpen">

<sequence>

<element name="servAttrCom" type="sl:ServAttrCom"/>

<element name="support" type="sl:UserMgmtOpenSupport"/>

</sequence>

</complexType>

<complexType name="V2XCommMode">

<sequence>

<element name="servAttrCom" type="sl:ServAttrCom"/>

<element name="v2XMode" type="sl:V2XCommModelsV2XMode"/>

</sequence>

</complexType>

<complexType name="TermDensity">

<sequence>

<choice minOccurs="1" maxOccurs="1">

<element name="servAttrCom" type="sl:ServAttrCom"/>

<element name="density" type="integer"/>

</choice>

</sequence>

</complexType>

<complexType name="ServiceProfile">

<sequence>

<element name="serviceProfileId" type="string"/>

<element name="sNSSAIList" type="ngc:SnssaiList"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="maxNumberofUEs" type="long" minOccurs="0"/>

<element name="latency" type="integer" minOccurs="0"/>

<element name="uEMobilityLevel" type="integer" minOccurs="0"/>

<element name="resourceSharingLevel" type="integer" minOccurs="0"/>

<element name="sst" type="ngc:Sst"/>

<element name="availability" type="float" minOccurs="0"/>

<element name="delayTolerance" type="sl:DelayTolerance" minOccurs="0"/>

<element name="deterministicComm" type="sl:DeterminComm" minOccurs="0"/>

<element name="dLThptPerSlice" type="sl:DLThpt" minOccurs="0"/>

<element name="dLThptPerUE" type="sl:DLThpt" minOccurs="0"/>

<element name="uLThptPerSlic" type="sl:ULThpt" minOccurs="0"/>

<element name="uLThptPerUE" type="sl:ULThpt" minOccurs="0"/>

<element name="maxPktSize" type="sl:MaxPktSize" minOccurs="0"/>

<element name="maxNumberofConns" type="sl:MaxNumberofConns" minOccurs="0"/>

<element name="kPIMonitoring" type="sl:KPIMonitoring" minOccurs="0"/>

<element name="userMgmtOpen" type="sl:UserMgmtOpen" minOccurs="0"/>

<element name="v2XCommModels" type="sl:V2XCommMode" minOccurs="0"/>

<element name="coverageArea" type="string" minOccurs="0"/>

<element name="termDensity" type="sl:TermDensity" minOccurs="0"/>

<element name="activityFactor" type="float" minOccurs="0"/>

<element name="uESpeed" type="integer" minOccurs="0"/>

<element name="jitter" type="integer" minOccurs="0"/>

<element name="survivalTime" type="string" minOccurs="0"/>

<element name="reliability" type="string" minOccurs="0"/>

</sequence>

</complexType>

<complexType name="ServiceProfileList">

<sequence>

<element name="serviceProfile" type="sl:ServiceProfile"/>

</sequence>

</complexType>

<complexType name="SliceProfile">

<sequence>

<element name="sliceProfileId" type="string"/>

<element name="sNSSAIList" type=" ngc:SnssaiList"/>

<element name="pLMNIdList" type="en:PLMNIdList"/>

<element name="perfReq" type="sl:PerfReq"/>

<element name="maxNumberofUEs" type="long" minOccurs="0"/>

<element name="coverageAreaTAList" type="ngc:NrTACList" minOccurs="0"/>

<element name="latency" type="integer" minOccurs="0"/>

<element name="uEMobilityLevel" type="sl:MobilityLevel" minOccurs="0"/>

<element name="resourceSharingLevel" type="integer" minOccurs="0"/>

</sequence>

</complexType>

<complexType name="SliceProfileList">

<sequence>

<element name="sliceProfile" type="sl:SliceProfile"/>

</sequence>

</complexType>

<complexType name="NsInfo">

<!-- Refer to definitions in clause 8.3.3.2.2 of ETSI NFV IFA013 -->

<sequence>

<element name="nsInstanceId" type="string"/>

<element name="nsName" type="string"/>

<element name="description" type="string"/>

</sequence>

</complexType>

<element name="NetworkSlice" substitutionGroup="xn:SubNetworkOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<!-- Inherited attributes from SubNetwork -->

<element name="dnPrefix" type="string" minOccurs="0"/>

<element name="userLabel" type="string"/>

<element name="userDefinedNetworkType" type="string"/>

<element name="setOfMcc" type="string" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<!-- End of inherited attributes from SubNetwork -->

<element name="operationalState" type="sm:operationalStateType"/>

<element name="administrativeState" type="sm:administrativeStateType"/>

<element name="serviceProfileList" type="sl:ServiceProfileList"/>			<element name="networkSliceSubnetRef" type="xn:dn"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="NetworkSliceSubnet" substitutionGroup="xn:SubNetworkOptionallyContainedNrmClass">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes">

<complexType>

<all>

<!-- Inherited attributes from SubNetwork -->

<element name="dnPrefix" type="string" minOccurs="0"/>

<element name="userLabel" type="string"/>

<element name="userDefinedNetworkType" type="string"/>

<element name="setOfMcc" type="string" minOccurs="0"/>

<element name="measurements" type="xn:MeasurementTypesAndGPsList" minOccurs="0"/>

<!-- End of inherited attributes from SubNetwork -->

<element name="operationalState" type="sm:operationalStateType"/>

<element name="administrativeState" type="sm:administrativeStateType"/>

<element name="nsInfo" type="sl:NsInfo" minOccurs="0"/>

<element name="sliceProfileList" type="sl:SliceProfileList"/>

<element name="managedFunctionRef" type="xn:dnlist"/>

<element name="networkSliceSubnetRef" type="xn:dnlist"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:MeasurementControl"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

</schema>

Annex J (normative):
OpenAPI definition of the Slice NRM

# J.1	General

This annex contains the OpenAPI definition of the Slice NRM in YAML format.

The Information Service (IS) of the NR NRM is defined in clause 6.

Mapping rules to produce the OpenAPI definition based on the IS are defined in TS 32.160 [14].

# J.2	Void

# J.3	Void

# J.4	Solution Set (SS) definitions

## J.4.1	Void

## J.4.2	Void

## J.4.3	OpenAPI document "sliceNrm.yaml"

openapi: 3.0.1

info:

title: Slice NRM

version: 16.5.0

description: >-

OAS 3.0.1 specification of the Slice NRM

@ 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: TS 28.541 V16.4.0; 5G NRM, Slice NRM

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.541/

paths: {}

components:

schemas:

#------------ Type definitions ---------------------------------------------------

Float:

type: number

format: float

MobilityLevel:

type: string

enum:

- STATIONARY

- NOMADIC

- RESTRICTED MOBILITY

- FULLY MOBILITY

SharingLevel:

type: string

enum:

- SHARED

- NON-SHARED

PerfReqEmbb:

type: object

properties:

expDataRateDL:

type: number

expDataRateUL:

type: number

areaTrafficCapDL:

type: number

areaTrafficCapUL:

type: number

userDensity:

type: number

activityFactor:

type: number

PerfReqEmbbList:

type: array

items:

$ref: '#/components/schemas/PerfReqEmbb'

PerfReqUrllc:

type: object

properties:

cSAvailabilityTarget:

type: number

cSReliabilityMeanTime:

type: string

expDataRate:

type: number

msgSizeByte:

type: string

transferIntervalTarget:

type: string

survivalTime:

type: string

PerfReqUrllcList:

type: array

items:

$ref: '#/components/schemas/PerfReqUrllc'

PerfReq:

oneOf:

- $ref: '#/components/schemas/PerfReqEmbbList'

- $ref: '#/components/schemas/PerfReqUrllcList'

Category:

type: string

enum:

- CHARACTER

- SCALABILITY

Tagging:

type: string

enum:

- PERFORMANCE

- FUNCTION

- OPERATION

Exposure:

type: string

enum:

- API

- KPI

ServAttrCom:

type: object

properties:

category:

$ref: '#/components/schemas/Category'

tagging:

$ref: '#/components/schemas/Tagging'

exposure:

$ref: '#/components/schemas/Exposure'

Support:

type: string

enum:

- NOT SUPPORTED

- SUPPORTED

DelayTolerance:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

support:

$ref: '#/components/schemas/Support'

DeterministicComm:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

availability:

$ref: '#/components/schemas/Support'

periodicityList:

type: string

DLThptPerSlice:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

guaThpt:

$ref: '#/components/schemas/Float'

maxThpt:

$ref: '#/components/schemas/Float'

DLThptPerUE:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

guaThpt:

$ref: '#/components/schemas/Float'

maxThpt:

$ref: '#/components/schemas/Float'

ULThptPerSlice:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

guaThpt:

$ref: '#/components/schemas/Float'

maxThpt:

$ref: '#/components/schemas/Float'

ULThptPerUE:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

guaThpt:

$ref: '#/components/schemas/Float'

maxThpt:

$ref: '#/components/schemas/Float'

MaxPktSize:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

maxsize:

type: integer

MaxNumberofConns:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

nOofConn:

type: integer

KPIMonitoring:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

kPIList:

type: string

UserMgmtOpen:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

support:

$ref: '#/components/schemas/Support'

V2XCommModels:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

v2XMode:

$ref: '#/components/schemas/Support'

TermDensity:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

density:

type: integer

NsInfo:

type: object

properties:

nsInstanceId:

type: string

nsName:

type: string

ServiceProfileList:

type: object

additionalProperties:

type: object

properties:

snssaiList:

$ref: 'nrNrm.yaml#/components/schemas/SnssaiList'

plmnIdList:

$ref: 'nrNrm.yaml#/components/schemas/PlmnIdList'

maxNumberofUEs:

type: number

latency:

type: number

uEMobilityLevel:

$ref: '#/components/schemas/MobilityLevel'

sst:

$ref: 'nrNrm.yaml#/components/schemas/Sst'

resourceSharingLevel:

$ref: '#/components/schemas/SharingLevel'

availability:

type: number

delayTolerance:

$ref: '#/components/schemas/DelayTolerance'

deterministicComm:

$ref: '#/components/schemas/DeterministicComm'

dLThptPerSlice:

$ref: '#/components/schemas/DLThptPerSlice'

dLThptPerUE:

$ref: '#/components/schemas/DLThptPerUE'

uLThptPerSlice:

$ref: '#/components/schemas/ULThptPerSlice'

uLThptPerUE:

$ref: '#/components/schemas/ULThptPerUE'

maxPktSize:

$ref: '#/components/schemas/MaxPktSize'

maxNumberofConns:

$ref: '#/components/schemas/MaxNumberofConns'

kPIMonitoring:

$ref: '#/components/schemas/KPIMonitoring'

userMgmtOpen:

$ref: '#/components/schemas/UserMgmtOpen'

v2XModels:

$ref: '#/components/schemas/V2XCommModels'

coverageArea:

type: string

termDensity:

$ref: '#/components/schemas/TermDensity'

activityFactor:

$ref: '#/components/schemas/Float'

uESpeed:

type: integer

jitter:

type: integer

survivalTime:

type: string

reliability:

type: string

SliceProfileList:

type: object

additionalProperties:

type: object

properties:

snssaiList:

$ref: 'nrNrm.yaml#/components/schemas/SnssaiList'

plmnIdList:

$ref: 'nrNrm.yaml#/components/schemas/PlmnIdList'

perfReq:

$ref: '#/components/schemas/PerfReq'

maxNumberofUEs:

type: number

coverageAreaTAList:

$ref: '5gcNrm.yaml#/components/schemas/TACList'

latency:

type: number

uEMobilityLevel:

$ref: '#/components/schemas/MobilityLevel'

resourceSharingLevel:

$ref: '#/components/schemas/SharingLevel'

IpAddress:

oneOf:

- $ref: 'genericNrm.yaml#/components/schemas/Ipv4Addr'

- $ref: 'genericNrm.yaml#/components/schemas/Ipv6Addr'

#------------ Definition of concrete IOCs ----------------------------------------

NetworkSlice:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/SubNetwork-Attr'

- type: object

properties:

networkSliceSubnetRef:

$ref: 'genericNrm.yaml#/components/schemas/Dn'

operationalState:

$ref: 'genericNrm.yaml#/components/schemas/OperationalState'

administrativeState:

$ref: 'genericNrm.yaml#/components/schemas/AdministrativeState'

serviceProfileList:

$ref: '#/components/schemas/ServiceProfileList'

NetworkSliceSubnet:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/SubNetwork-Attr'

- type: object

properties:

managedFunctionRefList:

$ref: 'genericNrm.yaml#/components/schemas/DnList'

networkSliceSubnetRefList:

$ref: 'genericNrm.yaml#/components/schemas/DnList'

operationalState:

$ref: 'genericNrm.yaml#/components/schemas/OperationalState'

administrativeState:

$ref: 'genericNrm.yaml#/components/schemas/AdministrativeState'

nsInfo:

$ref: '#/components/schemas/NsInfo'

sliceProfileList:

$ref: '#/components/schemas/SliceProfileList'

EPTransport:

$ref: '#/components/schemas/EP_Transport-Multiple'

EP_Transport-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

type: object

properties:

ipAddress:

$ref: '#/components/schemas/IpAddress'

logicInterfaceId:

type: string

nextHopInfo:

type: string

qosProfile:

type: string

epApplicationRefs:

$ref: 'genericNrm.yaml#/components/schemas/DnList'

EP_Transport-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_Transport-Single'

#------------ Definitions in TS 28.541 for TS 28.532 -----------------------------

resources-sliceNrm:

oneOf:

- $ref: '#/components/schemas/NetworkSlice'

- $ref: '#/components/schemas/NetworkSliceSubnet'

- $ref: '#/components/schemas/EP_Transport-Single'

Annex K (normative):
Void

Annex L (normative): 
Relation of GSMA GST, ServiceProfile and SliceProfile

# L.1	General

This annex describes the relation between GSMA GST[50] and information model ServiceProfile and SliceProfile.

# L.2	GSMA GST, ServiceProfile and sliceProfile

The GSMA GST is used as the SLA information for the communication between the vertical industry and the communication service provider. The SLA requirements can be fulfilled from management aspect and control aspect in a coordinated way. The SLS includes ServiceProfile information model.

As shown in figure L.2.1, the GST [50] is translated and used as input to NRM ServiceProfile, the ServiceProfile can be translated to corresponding requirements for dedicated domains. For example, 5GC SliceProfile is used to carry 5GC domain requirements,  NG-RAN SliceProfile is used to carry NG-RAN domain requirements, and TN requirements are translated and provide to TN domain. Some of the information in 5GC SliceProfile  and NG-RAN SliceProfile translated to configurable parameters  of network function for the control plane SLA support purpose.

NOTE:	how to do the translation is out of the scope of this document.

Figure L.2.1 Relation between GSMA GST, ServiceProfile and SliceProfile

Annex M (normative): 
Managed NF Service state handling

# M.1	Combined state diagram for a Managed NF Service

Figure M.1-1: Combined Managed NF Service state diagram

Table M.1-1: The Managed NF Service state transition table

| Trigger number | The state transition events and actions |
| --- | --- |
| 1 | Event: Received information of deployment of a Network Function (NF) service. Action: Create a ManagedNFService instance (MSI) whose(Administrative/Operational/Registration) are set to Locked/Disabled/Deregistered. |
| 2 | Event: Received information of positive state change of the NF service. Action: Set the Operational state of the MSI to Enabled. |
| 3 | Event: Received CM operation to unlock the NF Service or the NF. Action: Set the Administrative state of the MSI to Unlocked. Note: Changing Administrative state on NF service level is optional |
| 4 | Event: Received information that the NF Service is registered to an NRF either by the NF itself or by an OAM system on behalf of the NF. Action: Set the registration state of the MSI to Registered. |
| 5 | Event: Received information that the NF Service is deregistered from the NRF either by the NF itself or by an OAM system on behalf of the NF. Action: Set registration state of the MSI to Deregistered. |
| 6 | Event: Received information that the NF Service is unavailable because of, for example, limitation of resource or other exceptions. Action: Set the Operational state of the MSI to Disabled. |
| 7 | Event: Received information that the NF Service is unavilable. Action: Deregister the NF Service on behalf of the NF, and set the registration state of the MSI to Deregistered. |
| 8 | Event: Received CM operation to lock the NF Service or the NF. Action: Set the Administrative state of the MSI to Locked. Note: Changing Administrative state on NF service level is optional |
| 9 | Event: Received information that the NF Service is terminated or deleted, Action: Delete the MSI and set its state to NULL. |

Annex N (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2018-09 | SA#81 |  |  |  |  | Upgrade to change control version | 15.0.0 |
| 2018-09 | SA#81 |  |  |  |  | EdiHelp review | 15.0.1 |
| 2018-12 | SA#82 | SP-181046 | 0001 | 1 | F | Fix issues raised by EditHelp | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0002 | 2 | F | Update NR Stage 2 definition to align with TS 37.340 for MR-DC | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0003 | 1 | F | Update NRM Stage 2 defintion to align with TS 23.501 for 5G architecture | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0005 | 1 | F | Update Stage 3 XML definition of NR to align with Stage 2 content | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0006 | 1 | F | Update Stage 3 JSON definition of NR to align with Stage 2 content | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0007 | 1 | F | Update Stage 3 YANG definition of NR to align with Stage 2 content | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0008 | 1 | F | Update Stage 3 XML definition of 5GC to align with Stage 2 content | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0009 | 1 | F | Update Stage 3 JSON definition of 5GC to align with Stage 2 content | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0011 | 1 | F | Update stage 3 XML definition of NS to align with Stage 2 content | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0012 | 1 | F | Update Stage 3 JSON definition of NS to align with Stage 2 content | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0013 | 1 | F | Update stage 3 YANG definition of NS to align with Stage 2 content | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0014 | 1 | F | Correct the term sNSSAIList and nRTAClist | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0015 | 1 | F | Update the inheritance hierarchy figure for NR NRM to include BWP IOC and NRSectorCarrier IOC | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0016 | 1 | F | Change the term nCGI to nCI | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0019 | 1 | F | Align properties of cell state | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0021 | 1 | F | Add missing attribute definition and condition | 15.1.0 |
| 2018-12 | SA#82 | SP-181047 | 0022 | 1 | F | Add missing detail definition for attribute | 15.1.0 |
| 2018-12 | SA#82 | SP-181047 | 0023 | 1 | F | Adding missing attribute, and correction of reference | 15.1.0 |
| 2018-12 | SA#82 | SP-181043 | 0025 | - | F | Remove NSSF from the abbrevations | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0027 | - | F | Replace symbol for network slice state management | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0031 | 1 | F | Remove the ExternalENBFunction definition | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0033 | 1 | F | Align the management of external function and cell with TS 28.658 | 15.1.0 |
| 2018-12 | SA#82 | SP-181156 | 0034 | 1 | F | Update NR NRM with Cell Relation | 15.1.0 |
| 2018-12 | SA#82 | SP-181156 | 0038 | 3 | F | RRM Policy enhancements | 15.1.0 |
| 2018-12 | SA#82 | SP-181156 | 0039 | 1 | F | Fix containment issue in YANG definition | 15.1.0 |
| 2018-12 | SA#82 | SP-181156 | 0040 | - | F | Implement minor corrections | 15.1.0 |
| 2018-12 | SA#82 | SP-181042 | 0041 | - | F | Update Stage 3 NRM for RRM Policy enhancements | 15.1.0 |
| 2019-03 | SA#83 | SP-190121 | 0043 | 1 | F | Align NR attributes definition related to SSB with corresponding NG-RAN IE definition | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0044 | 1 | F | Correct the use of nCI and PLMN | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0045 | - | F | Remove duplicate definition for ExternalNRCellCU | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0046 | 2 | F | Correct class diagram for view on external entities | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0047 | 1 | F | Correct the definition for resourceSharingLevel | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0048 | 1 | F | Correction of references | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0052 | 1 | F | Align the term mFIdList and constituentNSSIIdList | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0053 | 1 | F | Correct the definition of nSSIId | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0054 | 1 | F | Add missing attribute constraint for class definition of NSSFFunction | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0055 | 1 | F | Correct attribute constraints for RRMpolicy related attributes in NRCellCU | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0057 | - | F | Correct cardinality of End Point (EP) to target | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0058 | 0 | F | Correct Import table | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0059 | - | F | Remove ExternalNRCellCU.pLMNIdList | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0060 | - | F | Use 'bS' (not 'bs') to prefix all BS (base station) attributes | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0061 | 1 | F | Correction of State attributes descriptions | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0062 | - | F | Update 5G JSON Solution Set to align with generic NRM | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0063 | 1 | F | Update YANG Solution Set to align with Stage 2 definition | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0064 | 1 | F | Update Information Service to fix Network Slice modeling issue | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0065 | 1 | F | Update Solution Set to fix Network Slice modeling issue | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0066 | 1 | F | Add availability in service profile of network slice resource model | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0068 | 1 | F | Add sST attribute to ServiceProfile | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0069 | 1 | F | Update to sST attribute stage 3 | 15.2.0 |
| 2019-03 | SA#83 | SP-190149 | 0073 | 2 | F | Replace CoverageAreaTAList type definition | 16.0.0 |
| 2019-03 | SA#83 | SP-190149 | 0074 | 1 | F | Name datatypes SliceProfile and ServiceProfile | 16.0.0 |
| 2019-03 | SA#83 | SP-190149 | 0075 | 1 | F | Add datatype definition for S-NSSAI | 16.0.0 |
| 2019-03 | SA#83 | SP-190149 | 0076 | 1 | F | Remove incomplete description for TAC | 16.0.0 |
| 2019-03 | SA#83 | SP-190149 | 0079 | 1 | F | Name datatype RRMPolicyRatio2 | 16.0.0 |
| 2019-06 | SA#84 | SP-190374 | 0083 | - | A | Remove attribute availabilityStatus in NRCellDU IOC | 16.1.0 |
| 2019-06 | SA#84 | SP-190373 | 0085 | 1 | F | Correct the definition for nsInfo | 16.1.0 |
| 2019-06 | SA#84 | SP-190374 | 0088 | 1 | A | Update Information Service of NR to fix unclear Note issue | 16.1.0 |
| 2019-06 | SA#84 | SP-190373 | 0096 | 2 | A | Correct the use of plmnIdList | 16.1.0 |
| 2019-06 | SA#84 | SP-190373 | 0098 | 1 | F | Add missing clauses to RRMPolicyRatio2 data type | 16.1.0 |
| 2019-06 | SA#84 | SP-190373 | 0099 | 1 | F | Update RRMPolicyRatio2 data type name in stage 3 | 16.1.0 |
| 2019-06 | SA#84 | SP-190373 | 0102 | - | F | Fix the implementation errors | 16.1.0 |
| 2019-09 | SA#85 | SP-190745 | 0089 | 2 | B | Update 5GC Information Service to align with Managed Service Definition | 16.2.0 |
| 2019-09 | SA#85 | SP-190743 | 0107 | 1 | A | Correct description for NR deployment scenario | 16.2.0 |
| 2019-09 | SA#85 | SP-190743 | 0109 | 1 | A | Correct NR NRM model to be applicable for all NG-RAN architecture | 16.2.0 |
| 2019-09 | SA#85 | SP-190745 | 0114 | 1 | C | Support NF Profile management | 16.2.0 |
| 2019-09 | SA#85 | SP-190743 | 0121 | 1 | A | Clarification of sNSSAIList attribute | 16.2.0 |
| 2019-09 | SA#85 | SP-190744 | 0123 | - | A | Remove pLMNId from GNBDUFunction | 16.2.0 |
| 2019-09 | SA#85 | SP-190743 | 0126 | 2 | A | Update class definition with inheritance information | 16.2.0 |
| 2019-09 | SA#85 | SP-190743 | 0128 | 1 | A | Correct description of NRCellCU and NRCellDU to be applicable for all deployment scenarios | 16.2.0 |
| 2019-09 | SA#85 | SP-190743 | 0130 | - | A | Correct XML solution set for NR | 16.2.0 |
| 2019-09 | SA#85 | SP-190743 | 0132 | - | A | Correct XML solution set for Network slice | 16.2.0 |
| 2019-09 | SA#85 | SP-190750 | 0133 | 1 | F | Clarification on slice model | 16.2.0 |
| 2019-09 | SA#85 | SP-190743 | 0142 | 1 | A | Add YANG mount info | 16.2.0 |
| 2019-09 | SA#85 | SP-190743 | 0143 | - | A | Add YANG solution | 16.2.0 |
| 2019-09 | SA#85 | SP-190745 | 0149 | 1 | F | generate JSON definition for 5GC NRM based on new style guideline | 16.2.0 |
| 2019-09 | SA#85 | SP-190744 | 0150 | 1 | A | Fix NR NRM to add missed ID info | 16.2.0 |
| 2019-09 | SA#85 | SP-190744 | 0152 | - | F | XML Solution Set for 5GC | 16.2.0 |
| 2019-09 | SA#85 | SP-190744 | 0154 | - | A | Correct ETSI NFV reference | 16.2.0 |
| 2019-09 | SA#85 | SP-190744 | 0157 | 1 | A | generate JSON definition for Slice NRM based on new style guideline | 16.2.0 |
| 2019-09 | SA#85 | SP-190744 | 0158 | 1 | A | generate JSON definition for NR NRM based on new style guideline | 16.2.0 |
| 2019-12 | SA#86 | SP-191159 | 0146 | 3 | F | To syn up with v1540 stage 2 | 16.3.0 |
| 2019-12 | SA#86 | SP-191173 | 0156 | 2 | A | Correct Import table | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0161 | 1 | C | Extensions to PCF and UPF IOCs for support of TSC (Time Sensitive Communication) | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0166 | 1 | F | Correct XML solution set for NR | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0167 | 1 | F | Correct Network slice NRM | 16.3.0 |
| 2019-12 | SA#86 | SP-191173 | 0168 | 2 | A | Correct NR TAC attribute property | 16.3.0 |
| 2019-12 | SA#86 | SP-191173 | 0170 | - | A | Correction of the duplicated IOC NSSFFunction in daigram | 16.3.0 |
| 2019-12 | SA#86 | SP-191173 | 0172 | - | A | Correction of the wrong IOC names in transport view diagram---Not implemented, wrong baseline (MCC) | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0175 | 2 | F | XML Solution Set for 5GC | 16.3.0 |
| 2019-12 | SA#86 | SP-191170 | 0177 | 3 | C | Update on slice NRM | 16.3.0 |
| 2019-12 | SA#86 | SP-191170 | 0178 | 2 | B | Add relation of GST and profiles | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0180 | 3 | F | Update SEPP Stage 2 definition in 5GC NRM | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0182 | 1 | C | Add NEF Stage 2 definition in 5GC NRM | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0184 | 1 | C | Add SCP Stage 2 definition in 5GC NRM | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0185 | - | C | Add Stage 3 definitions of 5GC NRM to align with stage 2 | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0186 | 1 | C | Support communication model in 5GC NF - Stage 2 | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0192 | 1 | F | Fix merging errors of the specification | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0195 | - | C | Add State Handling diagram for NF service | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0197 | - | B | Updates to YANG SS | 16.3.0 |
| 2019-12 | SA#86 | SP-191170 | 0198 | 1 | C | Update XML definitions of ServiceProfile NRM | 16.3.0 |
| 2019-12 | SA#86 | SP-191170 | 0199 | 2 | C | Update JSON definitions of ServiceProfile NRM | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0200 | 1 | C | Add managedNFProfile definition for ngc NRM - stage3 | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0202 | 2 | B | Add the RIM monitoring parameters for remote interference management | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0212 | 2 | F | Correct Network slice NRM | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0213 | - | F | Update SEPP Stage 3 definition in 5GC NRM | 16.3.0 |
| 2019-12 | SA#86 | SP-191180 | 0222 | 2 | B | Management of NR ANR, Stage 2 | 16.3.0 |
| 2019-12 | SA#86 | SP-191180 | 0223 | - | B | Management of NR ANR, Stage 3 | 16.3.0 |
| 2019-12 | SA#86 | SP-191173 | 0226 | 1 | A | Add Stages 2 NRM Info Model definitions for beam managed object classes | 16.3.0 |
| 2019-12 | SA#86 | SP-191173 | 0227 | - | A | Add Stages 2 NRM Info Model definitions for beam managed object classes | 16.3.0 |
| 2020-03 | SA#87E | SP-200169 | 0163 | 4 | F | Correct the parameter sNSSAIList | 16.4.0 |
| 2020-03 | SA#87E | SP-200169 | 0179 | 3 | C | Update of RRM Policy | 16.4.0 |
| 2020-03 | SA#87E | SP-200169 | 0235 | - | F | Correction of reference | 16.4.0 |
| 2020-03 | SA#87E | SP-200169 | 0239 | 1 | F | Update the NR NRM to align with NG-RAN overview architecture | 16.4.0 |
| 2020-03 | SA#87E | SP-200169 | 0241 | - | F | Some correction on the NR NRM | 16.4.0 |
| 2020-03 | SA#87E | SP-200169 | 0242 | - | F | Fix merging errors of the specification | 16.4.0 |
| 2020-03 | SA#87E | SP-200169 | 0243 | 1 | F | Update NRM attribute definitions | 16.4.0 |
| 2020-03 | SA#87E | SP-200233 | 0245 | 2 | B | Add the RIM parameters for remote interference management | 16.4.0 |
| 2020-03 | SA#87E | SP-200234 | 0248 | 1 | F | Update on slice NRM and solution sets | 16.4.0 |
| 2020-03 | SA#87E | SP-200234 | 0250 | 1 | F | Update of GNBCUUPFunction NRM | 16.4.0 |
| 2020-03 | SA#87E | SP-200232 | 0253 | 2 | B | Add Stage 3 NRM Info Model definitions for RRMPolicy and PLMNInfo related CRs | 16.4.0 |
| 2020-03 | SA#87E | SP-200178 | 0254 | 1 | F | Correct CR implementation errors | 16.4.0 |
| 2020-03 | SA#87E | SP-200235 | 0255 | 1 | F | Add OpenAPI definitions required by the ProvMnS | 16.4.0 |
| 2020-03 | SA#87E | SP-200169 | 0258 |  | F | Correct errors in yang solution set | 16.4.0 |
| 2020-03 | SA#87E |  |  |  |  | Correction of implementation errrors | 16.4.1 |
| 2020-06 | SA#88-e | SP-200489 | 0259 | 1 | F | Update on the RRMpolicyRatio | 16.5.0 |
| 2020-06 | SA#88-e | SP-200493 | 0260 | - | F | Replace DN with better identifier for whitelists and blacklists management | 16.5.0 |
| 2020-06 | SA#88-e | SP-200603 | 0261 | 1 | B | Add IOC for control of QoS monitoring per QoS flow per UE | 16.5.0 |
| 2020-06 | SA#88-e | SP-200604 | 0262 | 1 | B | Add IOC for control of GTP-U path QoS monitoring | 16.5.0 |
| 2020-06 | SA#88-e | SP-200489 | 0263 | 1 | F | Correction of reference | 16.5.0 |
| 2020-06 | SA#88-e | SP-200493 | 0268 | - | B | ANR management for EN-DC architecture | 16.5.0 |
| 2020-06 | SA#88-e | SP-200484 | 0269 | 1 | F | Clarification on network slice related identifiers | 16.5.0 |
| 2020-06 | SA#88-e | SP-200484 | 0270 | - | F | Stage 3 update for clarification on network slice related identifiers | 16.5.0 |
| 2020-06 | SA#88-e | SP-200484 | 0274 | 1 | F | Correct sNSSAI definition in XML solution set | 16.5.0 |
| 2020-06 | SA#88-e | SP-200484 | 0275 | 1 | F | Clarify the NR NRM used for different deployment scenarios | 16.5.0 |
| 2020-06 | SA#88-e | SP-200484 | 0278 | - | F | Add missing notification types to the definition of common notifications | 16.5.0 |
| 2020-06 | SA#88-e | SP-200491 | 0279 | 1 | A | Update on NRCellDU | 16.5.0 |
| 2020-06 | SA#88-e | SP-200491 | 0281 | 1 | A | Update Clause 4.2.1.2 Inheritance UML diagram | 16.5.0 |
| 2020-06 | SA#88-e | SP-200490 | 0283 | 2 | B | new NRM fragment to support RIM stage 2 | 16.5.0 |
| 2020-06 | SA#88-e | SP-200490 | 0284 | 1 | B | new NRM fragment to support RIM stage 3 | 16.5.0 |
| 2020-06 | SA#88-e | SP-200489 | 0285 | - | F | Update stage 3 on the RRMpolicyRatio | 16.5.0 |
| 2020-06 | SA#88-e | SP-200605 | 0286 | 2 | B | Add IOC for configurable 5QIs | 16.5.0 |
| 2020-06 | SA#88-e | SP-200490 | 0287 | 1 | B | Add IOC for 5QI to DSCP mapping | 16.5.0 |
| 2020-06 | SA#88-e | SP-200493 | 0289 | - | B | Stage3 add the NRM fragment for SON management | 16.5.0 |
| 2020-06 | SA#88-e | SP-200493 | 0290 | - | B | ANR management for EN-DC architecture | 16.5.0 |
| 2020-06 | SA#88-e | SP-200493 | 0291 | 1 | B | Add the NRM fragment for SON management | 16.5.0 |
| 2020-06 | SA#88-e | SP-200490 | 0293 | - | F | Add CommModelList NRM definition | 16.5.0 |
| 2020-06 | SA#88-e | SP-200490 | 0294 | 1 | F | Update NRM attribute definitions | 16.5.0 |
| 2020-06 | SA#88-e | SP-200490 | 0295 | 1 | F | Correct NRM definition in XML solution | 16.5.0 |
| 2020-06 | SA#88-e | SP-200485 | 0300 | 1 | F | Clarification on the relation of GST, ServiceProfile and SliceProfile | 16.5.0 |
| 2020-06 | SA#88-e | SP-200496 | 0301 | 1 | B | Add ES coverage relation in NRCellRelation | 16.5.0 |
| 2020-06 | SA#88-e | SP-200490 | 0302 | - | F | Update the decription for RRMPolicy_ and resouceType | 16.5.0 |
| 2020-06 | SA#88-e | SP-200490 | 0303 | - | F | Update definition for attribute localAddress in EP_RP IOC | 16.5.0 |
| 2020-06 | SA#88-e | SP-200486 | 0305 | 1 | A | Correction of references | 16.5.0 |
| 2020-06 | SA#88-e | SP-200485 | 0306 | 1 | F | add transport information and slice mapping on backhaul endpoints | 16.5.0 |
| 2020-06 | SA#88-e | SP-200485 | 0307 | - | F | add transport information and slice mapping on backhaul endpoints stage 3 | 16.5.0 |
| 2020-06 | SA#88-e | SP-200490 | 0312 | 1 | F | Update SliceProfile attributes solution 1 | 16.5.0 |
| 2020-06 | SA#88-e | SP-200490 | 0315 | 1 | B | Add configuredMaxTxEIRP on NRSectorCarrier | 16.5.0 |
| 2020-06 | SA#88-e | SP-200490 | 0316 | - | B | Stage 3 Add configuredMaxTxEIRP on NRSectorCarrier | 16.5.0 |
| 2020-06 | SA#88-e | SP-200490 | 0318 | - | F | Update NRM YANG for 28.541 | 16.5.0 |
| 2020-06 | SA#88-e | SP-200496 | 0319 | - | B | Add ES coverage relation in NRCellRelation Stage 3 | 16.5.0 |
| 2020-06 | SA#88-e | SP-200612 | 0320 | 1 | F | Update openAPI for NRCellRelation and NRFreqRelation | 16.5.0 |
| 2020-09 | SA#89-e | SP-200729 | 0321 | - | F | Correction of NRM YANG errors | 16.6.0 |
| 2020-09 | SA#89-e | SP-200729 | 0322 | 1 | F | Correct on NR NRM | 16.6.0 |
| 2020-09 | SA#89-e | SP-200729 | 0323 | - | F | Correct the openAPI definition for NR NRM | 16.6.0 |
| 2020-09 | SA#89-e | SP-200730 | 0325 | - | A | Correct on frequency related IOC | 16.6.0 |
| 2020-09 | SA#89-e | SP-200729 | 0329 | 1 | B | Add IOC for predefined PCC rules | 16.6.0 |
| 2020-09 | SA#89-e | SP-200729 | 0330 | 2 | B | Add IOC for predefined PCC rules | 16.6.0 |
| 2020-09 | SA#89-e | SP-200729 | 0331 | - | B | Enable PCF to support configurable 5QIs | 16.6.0 |
| 2020-09 | SA#89-e | SP-200729 | 0332 | - | B | Add IOC for dynamic 5QIs - stage 2 | 16.6.0 |
| 2020-09 | SA#89-e | SP-200729 | 0333 | - | B | Add IOC for dynamic 5QIs - stage 3 | 16.6.0 |
| 2020-09 | SA#89-e | SP-200729 | 0334 | - | B | Add TCE mapping info in GNBCUCPFunction | 16.6.0 |
| 2020-09 | SA#89-e | SP-200729 | 0335 | - | B | Add TCE mapping info in openAPI solution | 16.6.0 |
| 2020-09 | SA#89-e | SP-200729 | 0336 | - | F | Add missing definitions for perfReq | 16.6.0 |
| 2020-09 | SA#89-e | SP-200754 | 0338 | 1 | F | Delete supportedAccessTech to align with GST | 16.6.0 |
| 2020-09 | SA#89-e | SP-200724 | 0339 | - | F | Correction on duplicated annex numbering | 16.6.0 |
| 2020-09 | SA#89-e | SP-200729 | 0345 | - | F | Update NRM attribute definitions | 16.6.0 |
| 2020-09 | SA#89-e | SP-200749 | 0362 | - | F | Deleting SupportedAccessTech - Stage 3 - XML | 16.6.0 |
| 2020-09 | SA#89-e | SP-200724 | 0368 | 1 | F | Add relation between transport and application level endpoints | 16.6.0 |
| 2020-09 | SA#89-e | SP-200724 | 0369 | - | F | Add relation between transport and application level endpoints stage 3 | 16.6.0 |
| 2020-09 | SA#89-e | SP-200729 | 0370 | 1 | F | Cleanup stage 2 editorial issue and stage 3 yaml error | 16.6.0 |
| 2020-09 | SA#89-e | SP-200749 | 0371 | - | F | Add clarifying note to ServiceProfile | 16.6.0 |
| 2020-11 |  |  |  |  |  | No technical changes, cleanup of watermarks, hidden text and custom XMl, etc | 16.6.1 |