| 3GPP TS 28.541 V17.2.1 (2021-04) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3 (Release 17 ) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	22

Introduction	23

1	Scope	24

2	References	24

3	Definitions of terms, symbols and abbreviations	26

3.1	Terms	26

3.2	Symbols	27

3.3	Abbreviations	27

4	Information model definitions for NR NRM	28

4.1	Imported and associated information	28

4.1.1	Imported information entities and local labels	28

4.1.2	Associated information entities and local labels	28

4.2	Class diagram	28

4.2.1	Class diagram for gNB and en-gNB	28

4.2.1.1	Relationships	28

4.2.1.2	Inheritance	34

4.3	Class definitions	36

4.3.1	GNBDUFunction	36

4.3.1.1	Definition	36

4.3.1.2	Attributes	37

4.3.1.3	Attribute constraints	37

4.3.1.4	Notifications	37

4.3.2	GNBCUCPFunction	37

4.3.2.1	Definition	37

4.3.2.2	Attributes	38

4.3.2.3	Attribute constraints	38

4.3.2.4	Notifications	38

4.3.3	GNBCUUPFunction	38

4.3.3.1	Definition	38

4.3.3.2	Attributes	39

4.3.3.3	Attribute constraints	39

4.3.3.4	Notifications	39

4.3.4	NRCellCU	39

4.3.4.1	Definition	39

4.3.4.2	Attributes	39

4.3.4.3	Void	40

4.3.4.4	Notifications	40

4.3.5	NRCellDU	40

4.3.5.1	Definition	40

4.3.5.2	Attributes	40

4.3.5.3	Attribute constraints	41

4.3.5.4	Notifications	41

4.3.6	NRSectorCarrier	41

4.3.6.1	Definition	41

4.3.6.2	Attributes	42

4.3.6.3	Attribute constraints	42

4.3.6.4	Notifications	42

4.3.7	BWP	42

4.3.7.1	Definition	42

4.3.7.2	Attributes	42

4.3.7.3	Attribute constraints	43

4.3.7.4	Notifications	43

4.3.8	EP_E1	43

4.3.8.1	Definition	43

4.3.8.2	Attributes	43

4.3.8.3	Attribute constraints	43

4.3.8.4	Notifications	43

4.3.9	EP_XnU	43

4.3.9.1	Definition	43

4.3.9.2	Attributes	43

4.3.9.3	Attribute constraints	44

4.3.9.4	Notifications	44

4.3.10	EP_NgC	44

4.3.10.1	Definition	44

4.3.10.2	Attributes	44

4.3.10.3	Attribute constraints	44

4.3.10.4	Notifications	44

4.3.11	EP_NgU	44

4.3.11.1	Definition	44

4.3.11.2	Attributes	44

4.3.11.3	Attribute constraints	45

4.3.11.4	Notifications	45

4.3.12	EP_F1C	45

4.3.12.1	Definition	45

4.3.12.2	Attributes	45

4.3.12.3	Attribute constraints	45

4.3.12.4	Notifications	45

4.3.13	EP_F1U	45

4.3.13.1	Definition	45

4.3.13.2	Attributes	45

4.3.13.3	Attribute constraints	46

4.3.13.4	Notifications	46

4.3.14	EP_S1U	46

4.3.14.1	Definition	46

4.3.14.2	Attributes	46

4.3.14.3	Attribute constraints	46

4.3.14.4	Notifications	46

4.3.15	EP_X2C	46

4.3.15.1	Definition	46

4.3.15.2	Attributes	46

4.3.15.3	Attribute constraints	46

4.3.15.4	Notifications	47

4.3.16	EP_X2U	47

4.3.16.1	Definition	47

4.3.16.2	Attributes	47

4.3.16.3	Attribute constraints	47

4.3.16.4	Notifications	47

4.3.17	EP_XnC	47

4.3.17.1	Definition	47

4.3.17.2	Attributes	47

4.3.17.3	Attribute constraints	47

4.3.17.4	Notifications	47

4.3.18	ExternalGNBCUCPFunction	48

4.3.18.1	Definition	48

4.3.18.2	Attributes	48

4.3.18.3	Attribute constraints	48

4.3.18.4	Notifications	48

4.3.19	ExternalGNBCUUPFunction	48

4.3.19.1	Definition	48

4.3.19.2	Attributes	48

4.3.19.3	Attribute constraints	48

4.3.19.4	Notifications	48

4.3.20	ExternalGNBDUFunction	48

4.3.20.1	Definition	48

4.3.20.2	Attributes	49

4.3.20.3	Attribute constraints	49

4.3.20.4	Notifications	49

4.3.21	ExternalUPFFunction	49

4.3.21.1	Definition	49

4.3.21.2	Attributes	49

4.3.21.3	Attribute constraints	49

4.3.21.4	Notifications	49

4.3.22	ExternalAMFFunction	49

4.3.22.1	Definition	49

4.3.22.2	Attributes	49

4.3.22.3	Attribute constraints	49

4.3.22.4	Notifications	50

4.3.23	Void	50

4.3.24	ENBFunction <<ProxyClass>>	50

4.3.24.1	Definition	50

4.3.24.2	Attributes	50

4.3.24.3	Attribute constraints	50

4.3.24.4	Notifications	50

4.3.25	GNBCUCPFunction <<ProxyClass>>	50

4.3.25.1	Definition	50

4.3.25.2	Attributes	50

4.3.25.3	Attribute constraints	50

4.3.25.4	Notifications	50

4.3.26	GNBCUUPFunction <<ProxyClass>>	50

4.3.26.1	Definition	50

4.3.26.2	Attributes	50

4.3.26.3	Attribute constraints	50

4.3.26.4	Notifications	51

4.3.27	GNBDUFunction <<ProxyClass>>	51

4.3.27.1	Definition	51

4.3.27.2	Attributes	51

4.3.27.3	Attribute constraints	51

4.3.27.4	Notifications	51

4.3.28	ServingGWFFunction <<ProxyClass>>	51

4.3.28.1	Definition	51

4.3.28.2	Attributes	51

4.3.28.3	Attribute constraints	51

4.3.28.4	Notifications	51

4.3.29	UPFFunction <<ProxyClass>>	51

4.3.29.1	Definition	51

4.3.29.2	Attributes	51

4.3.29.3	Attribute constraints	51

4.3.29.4	Notifications	52

4.3.30	AMFFunction <<ProxyClass>>	52

4.3.30.1	Definition	52

4.3.30.2	Attributes	52

4.3.30.3	Attribute constraints	52

4.3.30.4	Notifications	52

4.3.31	Void	52

4.3.32	NRCellRelation	52

4.3.32.1	Definition	52

4.3.32.2	Attributes	52

4.3.32.3	Attribute constraints	53

4.3.32.4	Notifications	53

4.3.33	NRFreqRelation	53

4.3.33.1	Definition	53

4.3.33.2	Attributes	53

4.3.33.3	Attribute constraints	54

4.3.33.4	Notifications	54

4.3.34	Void	54

4.3.35	ExternalNRCellCU	54

4.3.35.1	Definition	54

4.3.35.2	Attributes	54

4.3.35.3	Attribute constraints	54

4.3.35.4	Notifications	55

4.3.36	RRMPolicyRatio	55

4.3.36.1	Definition	55

4.3.36.2	Attributes	56

4.3.36.3	Attribute constraints	56

4.3.36.4	Notifications	56

4.3.37	S-NSSAI <<dataType>>	56

4.3.37.1	Definition	56

4.3.37.2	Attributes	56

4.3.37.3	Attribute constraints	56

4.3.37.4	Notifications	56

4.3.38	NRFrequency	56

4.3.38.1	Definition	56

4.3.38.2	Attributes	56

4.3.38.3	Attribute constraints	57

4.3.38.4	Notifications	57

4.3.39	CommonBeamformingFunction	57

4.3.39.1	Definition	57

4.3.39.2	Attributes	57

4.3.39.3	Attribute constraints	57

4.3.39.4	Notifications	57

4.3.40	Beam	57

4.3.40.1	Definition	57

4.3.40.2	Attributes	58

4.3.40.3	Attribute constraints	58

4.3.41	PLMNInfo <<dataType>>	58

4.3.41.1	Definition	58

4.3.41.2	Attributes	58

4.3.41.3	Attribute constraints	58

4.3.41.4	Notifications	58

4.3.42	RRMPolicyMember <<dataType>>	59

4.3.42.1	Definition	59

4.3.42.2	Attributes	59

4.3.42.3	Attribute constraints	59

4.3.42.4	Notifications	59

4.3.43	RRMPolicy_	59

4.3.43.1	Definition	59

4.3.43.2	Attributes	59

4.3.43.3	Attribute constraints	59

4.3.43.4	Notifications	59

4.3.44	RRMPolicyManagedEntity <<ProxyClass>>	60

4.3.44.1	Definition	60

4.3.44.2	Attributes	60

4.3.44.3	Attribute constraints	60

4.3.44.4	Notifications	60

4.3.45	GNBCUCPNeighbour <<ProxyClass>>	60

4.3.45.1	Definition	60

4.3.45.2	Attributes	60

4.3.45.3	Attribute constraints	60

4.3.45.4	Notifications	61

4.3.46	GNBCUUPNeighbour <<ProxyClass>>	61

4.3.46.1	Definition	61

4.3.46.2	Attributes	61

4.3.46.3	Attribute constraints	61

4.3.46.4	Notifications	61

4.3.47	MappingSetIDBackhaulAddress  <<dataType>>	62

4.3.47.1	Definition	62

4.3.47.2	Attributes	62

4.3.47.3	Attribute constraints	62

4.3.47.4	Notifications	62

4.3.48	BackhaulAddress  <<dataType>>	62

4.3.48.1	Definition	62

4.3.48.2	Attributes	62

4.3.48.3	Attribute constraints	62

4.3.48.4	Notifications	62

4.3.49	TAI  <<dataType>>	62

4.3.49.1	Definition	62

4.3.49.2	Attributes	62

4.3.49.3	Attribute constraints	62

4.3.49.4	Notifications	63

4.3.51	FrequencyDomainPara <<dataType>>	64

4.3.51.1	Definition	64

4.3.51.2	Attributes	64

4.3.51.3	Attribute constraints	64

4.3.51.4	Notifications	64

4.3.52	SequenceDomainPara  <<dataType>>	64

4.3.52.1	Definition	64

4.3.52.2	Attributes	64

4.3.52.3	Attribute constraints	64

4.3.52.4	Notifications	64

4.3.53	TimeDomainPara  <<dataType>>	64

4.3.53.1	Definition	64

4.3.53.2	Attributes	64

4.3.53.3	Attribute constraints	65

4.3.53.4	Notifications	65

4.3.54	RimRSReportConf <<dataType>>	65

4.3.54.1	Definition	65

4.3.54.2	Attributes	65

4.3.54.3	Attribute constraints	65

4.3.54.4	Notifications	65

4.3.55	RimRSReportInfo <<dataType>>	65

4.3.55.1	Definition	65

4.3.55.2	Attributes	66

4.3.55.3	Attribute constraints	66

4.3.55.4	Notifications	66

4.3.57	DANRManagementFunction	66

4.3.57.1	Definition	66

4.3.57.2	Attributes	67

4.3.57.3	Attribute constraints	67

4.3.57.4	Notifications	67

4.3.58	DESManagementFunction	67

4.3.58.1	Definition	67

4.3.58.2	Attributes	67

4.3.58.3	Attribute constraints	68

4.3.58.4	Notification	68

4.3.59	DRACHOptimizationFunction	68

4.3.59.1	Definition	68

4.3.59.2	Attributes	68

4.3.59.3	Attribute constraints	68

4.3.59.4	Notifications	68

4.3.60	DMROFunction	69

4.3.60.1	Definition	69

4.3.60.2	Attributes	69

4.3.60.3	Attribute constraints	69

4.3.60.4	Notifications	69

4.3.61	DPCIConfigurationFunction	69

4.3.61.1	Definition	69

4.3.61.2	Attributes	69

4.3.61.3	Attribute constraints	69

4.3.61.4	Notifications	69

4.3.62	CPCIConfigurationFunction	70

4.3.62.1	Definition	70

4.3.62.2	Attributes	70

4.3.62.3	Attribute constraints	70

4.3.62.4	Notifications	70

4.3.63	CESManagementFunction	70

4.3.63.1	Definition	70

4.3.63.2	Attributes	70

4.3.63.3	Attribute constraints	70

4.3.63.4	Notification	71

4.3.64	AddressWithVlan <<dataType>>	71

4.3.64.1	Definition	71

4.3.64.2	Attributes	71

4.3.64.3	Attribute constraints	71

4.3.64.4	Notifications	71

4.3.65	TceIDMappingInfo  <<dataType>>	71

4.3.65.1	Definition	71

4.3.65.2	Attributes	71

4.3.65.3	Attribute constraints	71

4.3.65.4	Notifications	71

4.4	Attribute definitions	72

4.4.1	Attribute properties	72

4.5	Common notifications	101

4.5.1	Alarm notifications	101

4.5.2	Configuration notifications	101

4.5.3	Threshold Crossing notifications	102

5	Information Model definitions for 5GC NRM	102

5.1	Imported information entities and local labels	102

5.2	Class diagram	102

5.2.1	Class diagram of 5GC NFs	102

5.2.1.1	Relationships	102

5.2.1.2	Inheritance	109

5.2.2	Class diagram of AMF Region/AMF Set	112

5.2.2.1	Relationships	112

5.2.2.2	Inheritance	113

5.3	Class definitions	113

5.3.1	AMFFunction	113

5.3.1.1	Definition	113

5.3.1.2	Attributes	113

5.3.1.3	Attribute constraints	114

5.3.1.4	Notifications	114

5.3.2	SMFFunction	114

5.3.2.1	Definition	114

5.3.2.2	Attributes	114

5.3.2.3	Attribute constraints	114

5.3.2.4	Notifications	114

5.3.3	UPFFunction	114

5.3.3.1	Definition	114

5.3.3.2	Attributes	114

5.3.3.3	Attribute constraints	115

5.3.3.4	Notifications	115

5.3.4	N3IWFFunction	115

5.3.4.1	Definition	115

5.3.4.2	Attributes	115

5.3.4.3	Attribute constraints	115

5.3.4.4	Notifications	115

5.3.5	PCFFunction	115

5.3.5.1	Definition	115

5.3.5.2	Attributes	115

5.3.5.3	Attribute constraints	116

5.3.5.4	Notifications	116

5.3.6	AUSFFunction	116

5.3.6.1	Definition	116

5.3.6.2	Attributes	116

5.3.6.3	Attribute constraints	116

5.3.6.4	Notifications	116

5.3.7	UDMFunction	116

5.3.7.1	Definition	116

5.3.7.2	Attributes	116

5.3.5.3	Attribute constraints	117

5.3.5.4	Notifications	117

5.3.8	UDRFunction	117

5.3.8.1	Definition	117

5.3.8.2	Attributes	117

5.3.8.3	Attribute constraints	117

5.3.8.4	Notifications	117

5.3.9	UDSFFunction	117

5.3.9.1	Definition	117

5.3.9.2	Attributes	117

5.3.9.3	Attribute constraints	118

5.3.9.4	Notifications	118

5.3.10	NRFFunction	118

5.3.10.1	Definition	118

5.3.10.2	Attributes	118

5.3.10.3	Attribute constraints	118

5.3.10.4	Notifications	118

5.3.11	NSSFFunction	118

5.3.11.1	Definition	118

5.3.11.2	Attributes	118

5.3.11.3	Attribute constraints	119

5.3.11.4	Notifications	119

5.3.12	AFFunction	119

5.3.12.1	Definition	119

5.3.13	DNFunction	119

5.3.13.1	Definition	119

5.3.14	SMSFFunction	119

5.3.14.1	Definition	119

5.3.14.2	Attributes	119

5.3.14.3	Attribute constraints	119

5.3.14.4	Notifications	119

5.3.15	LMFFunction	120

5.3.15.1	Definition	120

5.3.15.2	Attributes	120

5.3.15.3	Attribute constraints	120

5.3.15.4	Notifications	120

5.3.16	NGEIRFunction	120

5.3.16.1	Definition	120

5.3.16.2	Attributes	120

5.3.16.3	Attribute constraints	120

5.3.16.4	Notifications	120

5.3.17	SEPPFunction	120

5.3.17.1	Definition	120

5.3.17.2	Attributes	121

5.3.17.3	Attribute constraints	121

5.3.17.4	Notifications	121

5.3.18	NWDAFFunction	121

5.3.18.1	Definition	121

5.3.18.2	Attributes	121

5.3.18.3	Attribute constraints	121

5.3.18.4	Notifications	121

5.3.19	EP_N2	121

5.3.19.1	Definition	121

5.3.19.2	Attributes	121

5.3.19.3	Attribute constraints	122

5.3.19.4	Notifications	122

5.3.20	EP_N3	122

5.3.20.1	Definition	122

5.3.20.2	Attributes	122

5.3.20.3	Attribute constraints	122

5.3.20.4	Notifications	122

5.3.21	EP_N4	122

5.3.21.1	Definition	122

5.3.21.2	Attributes	122

5.3.21.3	Attribute constraints	122

5.3.21.4	Notifications	122

5.3.22	EP_N5	123

5.3.22.1	Definition	123

5.3.22.2	Attributes	123

5.3.22.3	Attribute constraints	123

5.3.22.4	Notifications	123

5.3.23	EP_N6	123

5.3.23.1	Definition	123

5.3.23.2	Attributes	123

5.3.23.3	Attribute constraints	123

5.3.23.4	Notifications	123

5.3.24	EP_N7	123

5.3.24.1	Definition	123

5.3.24.2	Attributes	123

5.3.24.3	Attribute constraints	124

5.3.24.4	Notifications	124

5.3.25	EP_N8	124

5.3.25.1	Definition	124

5.3.25.2	Attributes	124

5.3.25.3	Attribute constraints	124

5.3.25.4	Notifications	124

5.3.26	EP_N9	124

5.3.26.1	Definition	124

5.3.26.2	Attributes	124

5.3.26.3	Attribute constraints	124

5.3.26.4	Notifications	124

5.3.27	EP_N10	125

5.3.27.1	Definition	125

5.3.27.2	Attributes	125

5.3.27.3	Attribute constraints	125

5.3.27.4	Notifications	125

5.3.28	EP_N11	125

5.3.28.1	Definition	125

5.3.28.2	Attributes	125

5.3.28.3	Attribute constraints	125

5.3.28.4	Notifications	125

5.3.29	EP_N12	125

5.3.29.1	Definition	125

5.3.29.2	Attributes	125

5.3.29.3	Attribute constraints	126

5.3.29.4	Notifications	126

5.3.30	EP_N13	126

5.3.30.1	Definition	126

5.3.30.2	Attributes	126

5.3.30.3	Attribute constraints	126

5.3.30.4	Notifications	126

5.3.31	EP_N14	126

5.3.31.1	Definition	126

5.3.31.2	Attributes	126

5.3.31.3	Attribute constraints	126

5.3.31.4	Notifications	126

5.3.32	EP_N15	127

5.3.32.1	Definition	127

5.3.32.2	Attributes	127

5.3.32.3	Attribute constraints	127

5.3.32.4	Notifications	127

5.3.33	EP_N16	127

5.3.33.1	Definition	127

5.3.33.2	Attributes	127

5.3.33.3	Attribute constraints	127

5.3.33.4	Notifications	127

5.3.34	EP_N17	127

5.3.34.1	Definition	127

5.3.34.2	Attributes	127

5.3.34.3	Attribute constraints	128

5.3.34.4	Notifications	128

5.3.35	EP_N20	128

5.3.35.1	Definition	128

5.3.35.2	Attributes	128

5.3.35.3	Attribute constraints	128

5.3.35.4	Notifications	128

5.3.36	EP_N21	128

5.3.36.1	Definition	128

5.3.36.2	Attributes	128

5.3.36.3	Attribute constraints	128

5.3.36.4	Notifications	128

5.3.37	EP_N22	129

5.3.37.1	Definition	129

5.3.37.2	Attributes	129

5.3.37.3	Attribute constraints	129

5.3.37.4	Notifications	129

5.3.38	EP_N26	129

5.3.38.1	Definition	129

5.3.38.2	Attributes	129

5.3.38.3	Attribute constraints	129

5.3.38.4	Notifications	129

5.3.39	Void	129

5.3.40	Void	129

5.3.41	EP_S5C	129

5.3.41.1	Definition	129

5.3.41.2	Attributes	130

5.3.41.3	Attribute constraints	130

5.3.41.4	Notifications	130

5.3.42	EP_S5U	130

5.3.42.1	Definition	130

5.3.42.2	Attributes	130

5.3.42.3	Attribute constraints	130

5.3.42.4	Notifications	130

5.3.43	EP_Rx	130

5.3.43.1	Definition	130

5.3.43.2	Attributes	130

5.3.43.3	Attribute constraints	130

5.3.43.4	Notifications	131

5.3.44	EP_MAP_SMSC	131

5.3.44.1	Definition	131

5.3.44.2	Attributes	131

5.3.44.3	Attribute constraints	131

5.3.44.4	Notifications	131

5.3.45	EP_NLS	131

5.3.45.1	Definition	131

5.3.45.2	Attributes	131

5.3.45.3	Attribute constraints	131

5.3.45.4	Notifications	131

5.3.46	EP_NLG	131

5.3.46.1	Definition	131

5.3.46.2	Attributes	132

5.3.46.3	Attribute constraints	132

5.3.46.4	Notifications	132

5.3.47	EP_N27	132

5.3.47.1	Definition	132

5.3.47.2	Attributes	132

5.3.47.3	Attribute constraints	132

5.3.47.4	Notifications	132

5.3.48	EP_N31	132

5.3.48.1	Definition	132

5.3.48.2	Attributes	132

5.3.48.3	Attribute constraints	133

5.3.48.4	Notifications	133

5.3.49	ExternalNRFFunction	133

5.3.49.1	Definition	133

5.3.49.2	Attributes	133

5.3.49.3	Attribute constraints	133

5.3.49.4	Notifications	133

5.3.50	ExternalNSSFFunction	133

5.3.50.1	Definition	133

5.3.50.2	Attributes	133

5.3.50.3	Attribute constraints	133

5.3.50.4	Notifications	134

5.3.51	AMFSet	134

5.3.51.1	Definition	134

5.3.51.2	Attributes	134

5.3.51.3	Attribute constraints	134

5.3.51.4	Notifications	134

5.3.52	AMFRegion	134

5.3.52.1	Definition	134

5.3.52.2	Attributes	134

5.3.52.3	Attribute constraints	135

5.3.52.4	Notifications	135

5.3.53	ExternalAMFFunction	135

5.3.53.1	Definition	135

5.3.53.2	Attributes	135

5.3.53.3	Attribute constraints	135

5.3.53.4	Notifications	135

5.3.54	ManagedNFProfile <<dataType>>	135

5.3.54.1	Definition	135

5.3.54.2	Attributes	135

5.3.54.3	Attribute constraints	136

5.3.54.4	Notifications	136

5.3.55	HostAddr <<choice>>	136

5.3.55.1	Definition	136

5.3.56	NFInfo <<choice>>	136

5.3.56.1	Definition	136

5.3.57	UdmInfo <<dataType>>	137

5.3.57.1	Definition	137

5.3.57.2	Attributes	137

5.3.57.3	Attribute constraints	137

5.3.57.4	Notifications	137

5.3.58	AusfInfo <<dataType>>	137

5.3.58.1	Definition	137

5.3.58.2	Attributes	137

5.3.58.3	Attribute constraints	137

5.3.58.4	Notifications	137

5.3.59	UpfInfo <<dataType>>	137

5.3.59.1	Definition	137

5.3.59.2	Attributes	137

5.3.59.3	Attribute constraints	137

5.3.59.4	Notifications	138

5.3.60	AmfInfo <<dataType>>	138

5.3.60.1	Definition	138

5.3.60.2	Attributes	138

5.3.60.3	Attribute constraints	138

5.3.60.4	Notifications	138

5.3.61	Udrinfo <<dataType>>	138

5.3.61.1	Definition	138

5.3.61.2	Attributes	138

5.3.61.3	Attribute constraints	138

5.3.61.4	Notifications	138

5.3.62	EP_N32	138

5.3.62.1	Definition	138

5.3.62.2	Attributes	138

5.3.62.3	Attribute constraints	139

5.3.62.4	Notifications	139

5.3.63	ExternalSEPPFunction	139

5.3.63.1	Definition	139

5.3.63.2	Attributes	139

5.3.63.3	Attribute constraints	139

5.3.63.4	Notifications	139

5.3.64	SEPPFunction <<ProxyClass>>	139

5.3.64.1	Definition	139

5.3.64.2	Attributes	139

5.3.64.3	Attribute constraints	139

5.3.64.4	Notifications	140

5.3.65	NEFFunction	140

5.3.65.1	Definition	140

5.3.65.2	Attributes	140

5.3.65.3	Attribute constraints	140

5.3.65.4	Notifications	140

5.3.66	SCPFunction	140

5.3.67.1	Definition	140

5.3.67.2	Attributes	140

5.3.67.3	Attribute constraints	140

5.3.67.4	Notifications	140

5.3.68	SupportedFunction <<dataType>>	141

5.3.68.1	Definition	141

5.3.68.2	Attributes	141

5.3.68.3	Attribute constraints	141

5.3.68.4	Notifications	141

5.3.69	CommModel <<dataType>>	141

5.3.69.1	Definition	141

5.3.69.2	Attributes	141

5.3.69.3	Attribute constraints	141

5.3.69.4	Notifications	141

5.3.70	QFQoSMonitoringControl	141

5.3.70.1	Definition	141

5.3.70.2	Attributes	141

5.3.70.3	Attribute constraints	142

5.3.70.4	Notifications	142

5.3.71	QFDelayThresholdsType <<dataType>>	142

5.3.71.1	Definition	142

5.3.71.2	Attributes	142

5.3.71.3	Attribute constraints	142

5.3.71.4	Notifications	142

5.3.72	GtpUPathQoSMonitoringControl	142

5.3.72.1	Definition	142

5.3.72.2	Attributes	143

5.3.72.3	Attribute constraints	143

5.3.72.4	Notifications	143

5.3.73	GtpUPathDelayThresholdsType <<dataType>>	143

5.3.73.1	Definition	143

5.3.73.2	Attributes	143

5.3.73.3	Attribute constraints	143

5.3.73.4	Notifications	143

5.3.75	Configurable5QISet	144

5.3.75.1	Definition	144

5.3.75.2	Attributes	144

5.3.75.3	Attribute constraints	144

5.3.75.4	Notifications	144

5.3.76	FiveQICharacteristics <<dataType>>	144

5.3.76.1	Definition	144

5.3.76.2	Attributes	144

5.3.76.3	Attribute constraints	144

5.3.76.4	Notifications	144

5.3.77	PacketErrorRate <<dataType>>	145

5.3.77.1	Definition	145

5.3.77.2	Attributes	145

5.3.77.3	Attribute constraints	145

5.3.77.4	Notifications	145

5.3.78	FiveQiDscpMappingSet	145

5.3.78.1	Definition	145

5.3.78.2	Attributes	145

5.3.78.3	Attribute constraints	145

5.3.78.4	Notifications	145

5.3.79	FiveQiDscpMapping <<dataType>>	145

5.3.79.1	Definition	145

5.3.79.2	Attributes	146

5.3.79.3	Attribute constraints	146

5.3.79.4	Notifications	146

5.3.80	PredefinedPccRuleSet	146

5.3.80.1	Definition	146

5.3.80.2	Attributes	146

5.3.80.3	Attribute constraints	146

5.3.80.4	Notifications	146

5.3.81	PccRule <<dataType>>	146

5.3.81.1	Definition	146

5.3.81.2	Attributes	146

5.3.81.3	Attribute constraints	147

5.3.81.4	Notifications	147

5.3.82	FlowInformation <<dataType>>	147

5.3.82.1	Definition	147

5.3.82.2	Attributes	147

5.3.82.3	Attribute constraints	147

5.3.82.4	Notifications	148

5.3.83	EthFlowDescription <<dataType>>	148

5.3.83.1	Definition	148

5.3.83.2	Attributes	148

5.3.83.3	Attribute constraints	148

5.3.83.4	Notifications	148

5.3.84	QoSData <<dataType>>	148

5.3.84.1	Definition	148

5.3.84.2	Attributes	148

5.3.84.3	Attribute constraints	149

5.3.84.4	Notifications	149

5.3.85	ARP <<dataType>>	149

5.3.85.1	Definition	149

5.3.85.2	Attributes	149

5.3.85.3	Attribute constraints	149

5.3.85.4	Notifications	149

5.3.86	TrafficControlData <<dataType>>	149

5.3.86.1	Definition	149

5.3.86.2	Attributes	150

5.3.86.3	Attribute constraints	150

5.3.86.4	Notifications	150

5.3.87	RedirectInformation <<dataType>>	150

5.3.87.1	Definition	150

5.3.87.2	Attributes	150

5.3.87.3	Attribute constraints	150

5.3.87.4	Notifications	150

5.3.88	RouteToLocation <<dataType>>	151

5.3.88.1	Definition	151

5.3.88.2	Attributes	151

5.3.88.3	Attribute constraints	151

5.3.88.4	Notifications	151

5.3.89	RouteInformation <<dataType>>	151

5.3.89.1	Definition	151

5.3.89.2	Attributes	151

5.3.89.3	Attribute constraints	151

5.3.89.4	Notifications	151

5.3.90	UpPathChgEvent <<dataType>>	152

5.3.90.1	Definition	152

5.3.90.2	Attributes	152

5.3.90.3	Attribute constraints	152

5.3.90.4	Notifications	152

5.3.91	SteeringMode <<dataType>>	152

5.3.91.1	Definition	152

5.3.91.2	Attributes	152

5.3.91.3	Attribute constraints	152

5.3.91.4	Notifications	152

5.3.92	ConditionData  <<dataType>>	153

5.3.92.1	Definition	153

5.3.92.2	Attributes	153

5.3.92.3	Attribute constraints	153

5.3.92.4	Notifications	153

5.3.93	TscaiInputContainer  <<dataType>>	153

5.3.93.1	Definition	153

5.3.93.2	Attributes	153

5.3.93.3	Attribute constraints	153

5.3.93.4	Notifications	153

5.3.94	Dynamic5QISet	154

5.3.94.1	Definition	154

5.3.94.2	Attributes	154

5.3.94.3	Attribute constraints	154

5.3.94.4	Notifications	154

5.4	Attribute definitions	154

5.4.1	Attribute properties	154

5.5	Common notifications	173

5.5.1	Alarm notifications	173

5.5.2	Configuration notifications	174

5.5.3	Threshold Crossing notifications	174

6	Information model definitions for network slice NRM	174

6.1	Imported information entities and local labels	174

6.2	Class diagram	175

6.2.1	Relationships	175

6.2.2	Inheritance	176

6.3	Class definitions	176

6.3.1	NetworkSlice	176

6.3.1.1	Definition	176

6.3.1.2	Attributes	177

6.3.1.3	Attribute constraints	177

6.3.1.4	Notifications	177

6.3.2	NetworkSliceSubnet	177

6.3.2.1	Definition	177

6.3.2.2	Attributes	177

6.3.2.3	Attribute constraints	177

6.3.2.4	Notifications	177

6.3.3	ServiceProfile <<dataType>>	177

6.3.3.1	Definition	177

6.3.3.2	Attributes	178

6.3.3.3	Attribute constraints	178

6.3.3.4	Notifications	178

6.3.4	SliceProfile <<dataType>>	179

6.3.4.1	Definition	179

6.3.4.2	Attributes	179

6.3.4.3	Attribute constraints	179

6.3.4.4	Notifications	179

6.3.5	NsInfo <<dataType>>	180

6.3.5.1	Definition	180

6.3.5.2	Attributes	180

6.3.5.3	Attribute constraints	180

6.3.5.4	Notifications	180

6.3.6	ServAttrCom <<dataType>>	180

6.3.6.1	Definition	180

6.3.6.2	Attributes	180

6.3.6.3	Attribute constraints	180

6.3.6.4	Notifications	180

6.3.7	DelayTolerance<<dataType>>	180

6.3.7.1	Definition	180

6.3.7.2	Attributes	181

6.3.7.3	Attribute constraints	181

6.3.7.4	Notifications	181

6.3.7	DeterminComm <<dataType>>	181

6.3.7.1	Definition	181

6.3.7.2	Attributes	181

6.3.7.3	Attribute constraints	181

6.3.7.4	Notifications	181

6.3.8	DLThpt<<dataType>>	181

6.3.8.1	Definition	181

6.3.8.2	Attributes	181

6.3.8.3	Attribute constraints	181

6.3.8.4	Notifications	182

6.3.9	ULThpt<<dataType>>	182

6.3.9.1	Definition	182

6.3.9.2	Attributes	182

6.3.9.3	Attribute constraints	182

6.3.9.4	Notifications	182

6.3.10	MaxPktSize <<dataType>>	182

6.3.10.1	Definition	182

6.3.10.2	Attributes	182

6.3.10.3	Attribute constraints	182

6.3.10.4	Notifications	182

6.3.11	MaxNumberofPDUSessions <<dataType>>	182

6.3.11.1	Definition	182

6.3.11.2	Attributes	183

6.3.11.3	Attribute constraints	183

6.3.11.4	Notifications	183

6.3.12	Void	183

6.3.13	KPIMonitoring <<dataType>>	183

6.3.13.1	Definition	183

6.3.13.2	Attributes	183

6.3.13.3	Attribute constraints	183

6.3.13.4	Notifications	183

6.3.14	UserMgmtOpen<<dataType>>	183

6.3.14.1	Definition	183

6.3.14.2	Attributes	183

6.3.14.3	Attribute constraints	183

6.3.14.4	Notifications	183

6.3.15	V2XCommMode<<dataType>>	184

6.3.15.1	Definition	184

6.3.15.2	Attributes	184

6.3.15.3	Attribute constraints	184

6.3.15.4	Notifications	184

6.3.16	TermDensity<<dataType>>	184

6.3.16.1	Definition	184

6.3.16.2	Attributes	184

6.3.16.3	Attribute constraints	184

6.3.16.4	Notifications	184

6.3.17	EP_Transport	184

6.3.17.1	Definition	184

6.3.17.2	Attributes	184

6.3.17.3	Attribute constraints	185

6.3.17.4	Notifications	185

6.3.18	EP_Application <<ProxyClass>>	185

6.3.18.1	Definition	185

6.3.18.2	Attributes	185

6.3.18.3	Attribute constraints	185

6.3.18.4	Notifications	185

6.3.19	NBIoT <<dataType>>	185

6.3.19.1	Definition	185

6.3.19.2	Attributes	185

6.3.19.3	Attribute constraints	185

6.3.19.4	Notifications	185

6.3.20	DLThptSliceSubnet<<dataType>>	186

6.3.20.1	Definition	186

6.3.20.2	Attributes	186

6.3.20.3	Attribute constraints	186

6.3.20.4	Notifications	186

6.3.21	ULThptSliceSubnet<<dataType>>	186

6.3.21.1	Definition	186

6.3.21.2	Attributes	186

6.3.21.3	Attribute constraints	186

6.3.21.4	Notifications	186

6.3.22	CNSliceSubnetProfile<<dataType>>	186

6.3.22.1	Definition	186

6.3.22.2	Attributes	187

6.3.22.3	Attribute constraints	187

6.3.22.4	Notifications	187

6.3.23	RANSliceSubnetProfile<<dataType>>	187

6.3.23.1	Definition	187

6.3.23.2	Attributes	187

6.3.23.3	Attribute constraints	188

6.3.23.4	Notifications	188

6.3.24	TopSliceSubnetProfile<<dataType>>	188

6.3.24.1	Definition	188

6.3.24.2	Attributes	188

6.3.24.3	Attribute constraints	189

6.3.24.4	Notifications	189

6.3.25	Positioning <<dataType>>	189

6.3.25.1	Definition	189

6.3.25.2	Attributes	189

6.3.25.3	Attribute constraints	189

6.3.25.4	Notifications	189

6.3.26	Synchronicity <<dataType>>	189

6.3.26.1	Definition	189

6.3.26.2	Attributes	189

6.3.26.3	Attribute constraints	189

6.3.26.4	Notifications	190

6.4	Attribute definition	190

6.4.1	Attribute properties	190

6.5	Common notifications	200

6.5.1	Alarm notifications	200

6.5.2	Configuration notifications	200

6.5.3	Threshold Crossing notifications	200

7	Solution Set (SS)	200

Annex A (normative): Cell state handling	202

A.1	Relation between the administrative state and the "Pre-operation state of the gNB-DU Cell"	202

A.2	Combined state diagram for gNB cell	202

Annex B (normative):  NSI and NSSI state handling	206

B.1	NSI state handling	206

B.2	State handling of NSSI	207

Annex C (normative): Void	209

Annex D (normative): OpenAPI definition of the NR NRM	210

D.1	General	210

D.2	Void	210

D.3	Void	210

D.4	Solution Set (SS) definitions	210

D.4.1	Void	210

D.4.2	Void	210

D.4.3	OpenAPI document "nrNrm.yaml"	210

Annex E (normative): YANG definitions for NR NRM	235

E.1	General	235

E.2	Void	235

E.3	Void	235

E.4	Void	235

E.5	Modules	235

E.5.1	module _3gpp-nr-nrm-beam@2019-11-22.yang	235

E.5.1a	module _3gpp-nr-nrm-bwp.yang	237

E.5.1b	module _3gpp-nr-nrm-commonbeamformingfunction@2019-11-22.yang	238

E.5.2	module_3gpp-nr-nrm-ep.yang	239

E.5.3	module _3gpp-nr-nrm-eutrancellrelation@2019-10-28.yang	242

E.5.4	module _3gpp-nr-nrm-eutranetwork@2019-06-17.yang	244

E.5.5	module _3gpp-nr-nrm-eutranfreqrelation@2019-10-28.yang	245

E.5.6	module _3gpp-nr-nrm-eutranfrequency@2019-10-28.yang	248

E.5.7	module _3gpp-nr-nrm-externalamffunction@2019-10-28.yang	249

E.5.8	module _3gpp-nr-nrm-externalenbfunction@2019-10-28.yang	250

E.5.9	module_3gpp-nr-nrm-externaleutrancell@2019-10-28.yang	251

E.5.10	module _3gpp-nr-nrm-externalgnbcucpfunction@2019-10-28.yang	253

E.5.11	module _3gpp-nr-nrm-externalgnbcuupfunction@2019-10-28.yang	254

E.5.12	module _3gpp-nr-nrm-externalgnbdufunction@2019-10-28.yang	255

E.5.13	module _3gpp-nr-nrm-externalnrcellcu@2019-10-28.yang	256

E.5.14	module _3gpp-nr-nrm-externalservinggwfunction@2019-10-28.yang	260

E.5.15	module _3gpp-nr-nrm-externalupffunction@2019-10-28.yang	260

E.5.16	module _3gpp-nr-nrm-gnbcucpfunction.yang	261

E.5.17	module _3gpp-nr-nrm-gnbcuupfunction.yang	263

E.5.18	module_3gpp-nr-nrm-gnbdufunction.yang	265

E.5.19	module _3gpp-nr-nrm-nrcellcu.yang	266

E.5.20	module _3gpp-nr-nrm-nrcelldu.yang	268

E.5.21	module _3gpp-nr-nrm-nrcellrelation.yang	271

E.5.22	module _3gpp-nr-nrm-nrfreqrelation@2019-10-28.yang	273

E.5.23	module _3gpp-nr-nrm-nrfrequency@2019-10-28.yang	276

E.5.24	module _3gpp-nr-nrm-nrnetwork@2019-06-17.yang	277

E.5.25	module _3gpp-nr-nrm-nrsectorcarrier.yang	277

E.5.26	module _3gpp-nr-nrm-rrmpolicy.yang	279

E.5.27	Void	281

E.5.28	module _3gpp-nr-nrm-danrmanagementfunction.yang	281

E.5.29	module _3gpp-nr-nrm-desmanagementfunction.yang	281

E.5.30	module _3gpp-nr-nrm-drachoptimizationfunction.yang	285

E.5.31	module _3gpp-nr-nrm-dmrofunction.yang	287

E.5.32	module _3gpp-nr-nrm-dpciconfigurationfunction.yang	288

E.5.33	module _3gpp-nr-nrm-cpciconfigurationfunction.yang	289

E.5.34	module _3gpp-nr-nrm-cesmanagementfunction.yang	290

E.6	Void	291

E.7	Mount information	291

Annex F (normative): Void	292

Annex G (normative): OpenAPI definition of the 5GC NRM	293

G.1	General	293

G.2	Void	293

G.3	Void	293

G.4	Solution Set (SS) definitions	293

G.4.1	Void	293

G.4.2	Void	293

G.4.3	OpenAPI document "5gcNrm.yaml"	293

Annex H (normative): YANG definitions for 5GC	319

H.1	General	319

H.2	Void	319

H.3	Void	319

H.4	Void	319

H.5	Modules	319

H.5.1	module _3gpp-5gc-common-yang-types.yang	319

H.5.1a	module _3gpp-5gc-nrm-affunction@2019-10-28.yang	321

H.5.2	module _3gpp-5gc-nrm-amffunction.yang	321

H.5.3	module _3gpp-5gc-nrm-amfregion.yang	323

H.5.4	module _3gpp-5gc-nrm-amfset.yang	324

H.5.5	module _3gpp-5gc-nrm-ausffunction.yang	325

H.5.6	module _3gpp-5gc-nrm-dnfunction@2019-10-28.yang	326

H.5.7	module _3gpp-5gc-nrm-ep@2019-11-18.yang	327

H.5.8	module _3gpp-5gc-nrm-externalnrffunction@2019-10-28.yang	335

H.5.9	module _3gpp-5gc-nrm-externalnssffunction@2019-10-28.yang	336

H.5.10	module _3gpp-5gc-nrm-lmffunction@2019-10-25.yang	336

H.5.11	module _3gpp-5gc-nrm-n3iwffunction@2019-10-28.yang	337

H.5.12	module _3gpp-5gc-nrm-nfprofile@2019-06-17.yang	338

H.5.13	module _3gpp-5gc-nrm-nfservice.yang	353

H.5.14	module _3gpp-5gc-nrm-ngeirfunction.yang	357

H.5.15	module _3gpp-5gc-nrm-nrffunction.yang	358

H.5.16	module _3gpp-5gc-nrm-nssffunction.yang	359

H.5.17	module _3gpp-5gc-nrm-nwdaffunction.yang	360

H.5.18	module _3gpp-5gc-nrm-pcffunction.yang	361

H.5.19	module _3gpp-5gc-nrm-seppfunction.yang	363

H.5.19a	 module _3gpp-5gc-nrm- externalseppfunction@2019-11-17.yang	364

H.5.20	module _3gpp-5gc-nrm-smffunction.yang	364

H.5.21	module _3gpp-5gc-nrm-smsffunction@2019-10-25.yang	366

H.5.22	module _3gpp-5gc-nrm-udmfunction.yang	367

H.5.23	module _3gpp-5gc-nrm-udrfunction.yang	368

H.5.24	module _3gpp-5gc-nrm-udsffunction.yang	369

H.5.25	module _3gpp-5gc-nrm-upffunction.yang	370

H.5.26	module _3gpp-5gc-nrm-scpfunction.yang	371

H.5.27	module _3gpp-5gc-nrm-neffunction.yang	372

H.5.28	module _3gpp-5gc-nrm-QFQoSMonitoringControl.yang	373

H.5.29	module _3gpp-5gc-nrm-GtpUPathQoSMonitoringControl.yang	375

H.5.30	module _3gpp-5gc-nrm-Configurable5QISet.yang	376

H.5.31	module _3gpp-5gc-nrm-FiveQiDscpMappingSet.yang	378

H.5.32	module _3gpp-5gc-nrm-PredefinedPccRuleSet.yang	379

H.5.33	module _3gpp-5gc-nrm-dynamic5QISet@2020-08-06.yang	388

H.6	Void	388

H.7	Mount information	388

Annex I (normative): Void	390

Annex J (normative): OpenAPI definition of the Slice NRM	391

J.1	General	391

J.2	Void	391

J.3	Void	391

J.4	Solution Set (SS) definitions	391

J.4.1	Void	391

J.4.2	Void	391

J.4.3	OpenAPI document "sliceNrm.yaml"	391

Annex K (normative): Void	400

Annex L (normative):  Relation of GSMA GST, ServiceProfile and SliceProfile	401

L.1	General	401

L.2	GSMA GST, ServiceProfile and sliceProfile	401

Annex M (normative):  Managed NF Service state handling	403

M.1	Combined state diagram for a Managed NF Service	403

N.2	Modules	405

N.2.1	module _3gpp-ns-nrm-networkslice.yang	405

N.2.2	module _3gpp-ns-nrm-networkslicesubnet.yang	406

N.2.3	module _3gpp-ns-nrm-perfreq.yang	409

N.2.4	module _3gpp-ns-nrm-serviceprofile.yang	411

N.2.5	module _3gpp-ns-nrm-sliceprofile.yang	418

Annex O (informative): Change history	421

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

The present document is part of a TS-family covering the 3rd Generation Partnership Project Technical Specification Group Services and System Aspects Management and orchestration of networks, as identified below:

TS 28.540:	Management and orchestration of 5G networks; Network Resource Model (NRM); Stage 1.

TS 28.541:	Management and orchestration of 5G networks; Network Resource Model (NRM); Stage 2 and stage 3.

# 1	Scope

The present document specifies the Information Model and Solution Set for the Network Resource Model (NRM) definitions of NR, NG-RAN, 5G Core Network (5GC) and network slice, to fulfil the requirements identified in 3GPP TS 28.540 [10].

The Information Model defines the semantics and behaviour of information object class attributes and relations visible on the management interfaces in a protocol and technology neutral way. And Solution Set defines one or more solution set(s) with specific protocol(s) according to the Information Model definitions.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System".

[3]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[4]	3GPP TS 38.401: "NG-RAN; Architecture description".

[5]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[6]	3GPP TS 38.420: "NG-RAN; Xn general aspects and principles".

[7]	3GPP TS 38.470: "NG-RAN; F1 general aspects and principles".

[8]	3GPP TS 38.473: "NG-RAN; F1 application protocol (F1AP)".

[9]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage 2".

[10]	3GPP TS 28.540: "Management and orchestration; 5G Network Resource Model (NRM);Stage 1".

[11]	3GPP TS 28.662: "Telecommunication management; Generic Radio Access Network (RAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS) ".

[12]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[13]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[14]	3GPP TS 36.410: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 general aspects and principles".

[15]	3GPP TS 36.423: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 application protocol".

[16]	3GPP TS 36.425: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 interface user plane protocol".

[17]	3GPP TS 28.625: "State Management Data Definition Integration Reference Point (IRP); Information Service (IS)".

[18]	ITU-T Recommendation X.731: "Information technology - Open Systems Interconnection - Systems Management: State management function".

[19]	3GPP TS 28.658: "Telecommunications management; Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)".

[20]	3GPP TS 28.702: "Core Network (CN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[21]	3GPP TS 28.708: "Telecommunication management; Evolved Packet Core (EPC) Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)".

[22]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[23]	3GPP TS 29.510: "5G system; Network Function Repository Services; Stage 3".

[24]	3GPP TS 29.531: "5G System; Network Slice Selection Services Stage 3".

[25]	Void.

[26]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[27]	3GPP TS 28.554: "Management and orchestration; 5G End to end Key Performance Indicators (KPI)".

[28]	3GPP TS 22.261: "Service requirements for next generation new services and markets".

[29]	ETSI GS NFV-IFA 013 V2.4.1 (2018-02) "Network Function Virtualisation (NFV); Management and Orchestration; Os-Ma-nfvo Reference Point - Interface and Information Model Specification".

[30]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[31]	Void.

[32]	3GPP TS 38.211: "NR; Physical channels and modulation".

[33]	3GPP TS 32.616: "Telecommunication management; Configuration Management (CM); Bulk CM Integration Reference Point (IRP); Solution Set (SS) definitions".

[34]	3GPP TS 28.623: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

[35]	3GPP TS 28.532: "Management and orchestration; Management services".

[36]	Void.

[37]	IETF RFC 791: "Internet Protocol".

[38]	IETF RFC 2373: "IP Version 6 Addressing Architecture".

[39]	IEEE 802.1Q: "Media Access Control Bridges and Virtual Bridged Local Area Networks".

[40]	ETSI GR NFV-IFA 015 (V2.4.1): "Network Function Virtualisation (NFV) Release 2; Management and Orchestration; Report on NFV Information Model".

[41]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[42]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[43]	3GPP TS 32.156: "Telecommunication management; Fixed Mobile Convergence (FMC) model repertoire".

[44]	IETF RFC 4122: "A Universally Unique IDentifier (UUID) URN Namespace".

[45]	IETF RFC 8528: "YANG Schema Mount".

[46]	Void

[47]	3GPP TS 32.160: "Management and orchestration; Management Service Template".

[48]	3GPP TS 38.463: "NG-RAN; E1 application protocol (E1AP)".

[49]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in Idle mode and RRC Inactive state".

[50]	GSMA NG.116 - Generic Network Slice Template Version 3.0 (2020-05-22).

[51]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains; Stage 1".

[52]	3GPP TS 33.501: " Security architecture and procedures for the 5G System".

[53]	3GPP TS 38.901: "Study on channel model for frequencies from 0.5 to 100 GHz ".

[54]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[55]	3GPP TS 38.215: "NR; Physical layer measurements".

[56]	3GPP TS 29.244: "Technical Specification Group Core Network and Terminals; Interface between the Control Plane and the User Plane Nodes; Stage 3".

[57]	3GPP TS 28.313: "Self-Organizing Networks (SON) for 5G networks".

[58]	3GPP TS 38.423: "NR; Xn application protocol (XnAP)".

[59]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[60]	3GPP TS 29.512: "5G System; Session Management Policy Control Service; Stage 3".

[61]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[62]	3GPP TS 29.214: "Policy and Charging Control over Rx reference point".

[63]	IETF RFC 7042: "IANA Considerations and IETF Protocol and Documentation Usage for IEEE 802 Parameters".

[64]	IEEE 802.3-2015: "IEEE Standard for Ethernet".

[65]	IEEE 802.1Q-2014: "Bridges and Bridged Networks".

[66]	IETF RFC 4301: "Security Architecture for the Internet Protocol".

[67]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[68]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[69]	3GPP TS 28.552: " Management and orchestration; 5G performance measurements".

[70]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements ".

[71]	3GPP TS 28.310: " Management and orchestration; Energy efficiency of 5G".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1], 3GPP TS 28.540 [10] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1] and 3GPP TS 28.540 [10].

## 3.2	Symbols

void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1], 3GPP TS 23.501 [2], 3GPP TS 38.401 [4], 3GPP TS 28.540 [10] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1] , 3GPP TS 23.501 [2], 3GPP TS 38.401 [4] and 3GPP TS 28.540 [10].

BWP	Bandwidth part

CM	Configuration Management

DN	Distinguished Name

IOC	Information Object Class

JSON	JavaScript Object Notation

NFV	Network Functions Virtualisation

NRM	Network Resource Model

NS	Network Service

NSI	Network Slice Instance

NSSAI	Network Slice Selection Assistance Information

NSSI	Network Slice Subnet Instance

PNF	Physical Network Function

RIM	Remote interference management

RIM-RS	Remote interference management reference signal

SBA	Service Based Architecture

SS	Solution Set

TN	Transport Network

VNF	Virtualised Network Function

# 4	Information model definitions for NR NRM

## 4.1	Imported and associated information

### 4.1.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.622 [30], IOC, ManagedFunction | ManagedFunction |
| TS 28.622 [30], IOC, EP_RP | EP_RP |
| TS 28.662 [11], IOC, SectorEquipmentFunction | SectorEquipmentFunction |
| TS 28.658 [19], IOC, ExternalENBFunction | ExternalENBFunction |
| TS 28.708 [21], IOC, ServingGWFunction | ServingGWFunction |
| TS 28.658 [19], IOC, EUtranCellFDD | EUtranCellFDD |
| TS 28.658 [19], IOC, EUtranCellTDD | EUtranCellTDD |
| TS 28.658 [19], dataType, PLMNId | PLMNId |
| TS 28.658 [19], IOC, ENBFunction | ENBFunction |
| TS 28.708 [21], IOC, ExternalServingGWFunction | ExternalServingGWFunction |
| TS 28.658 [19], IOC, ExternalEUtranCellFDD | ExternalEUtranCellFDD |
| TS 28.658 [19], IOC, ExternalEUtranCellTDD | ExternalEUtranCellTDD |
| TS 28.658 [19], IOC, AdjacentCell | AdjacentEUtranCell |
| TS 28.658 [19], IOC, EUtranFrequency | EUtranFrequency |
| TS 28.658 [19], IOC, EUtranFreqRelation | EUtranFreqRelation |
| TS 28.658 [19], IOC, EUtranRelation | EUtranCellRelation |

### 4.1.2	Associated information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.622 [30], IOC, ManagedElement | ManagedElement |
| TS 28.622 [30], IOC, SubNetwork | SubNetwork |

## 4.2	Class diagram

### 4.2.1	Class diagram for gNB and en-gNB

#### 4.2.1.1	Relationships

This clause depicts the set of classes (e.g. IOCs) that encapsulates the information relevant for this gNB and en-gNB. For the UML semantics, see 3GPP TS 32.156 [43]. Subsequent clauses provide more detailed specification of various aspects of these classes.

The model fragments are for management representation of gNB and en-gNB for all NG-RAN deployment scenario as listed below.

-	Non-split NG-RAN deployment scenario, represents the gNB defined in TS 38.401[4]. In this scenario, a gNB is represented by a combination of a GNBCUCPFunction, one or more GNBCUUPFunctions and one or more GNBDUFunctions.

-	2-split NG-RAN deployment scenario, represents the gNB consist of gNB-CU and gNB-DU defined in TS 38.401[4] clause 6.1.1. In this scenario, a gNB-CU is represented by a combination of a GNBCUCPFunction and one or more GNBCUUPFunctions, whereas a gNB-DU is represented by a GNBDUFunction.

-	3-split NG-RAN deployment scenario, represents the gNB consist of gNB-CU-CP, gNB-CU-UP and gNB-DU defined in TS 38.401[4] clause 6.1.2. In this scenario, a gNB-CU-CP is represented by a GNBCUCPFunction, a gNB-CU-UP is represented by a GNBCUUPFunction, and a gNB-DU is represented by a GNBDUFunction.

Figure 4.2.1.1-1: NRM for all deployment scenarios

Figure 4.2.1.1-2: NRM for EPs for all deployment scenarios

Figure 4.2.1.1-3: NRM for <<IOC>>NRSectorCarrier and <<IOC>>BWP for all deployment scenarios

Figure 4.2.1.1-4: Cell Relation view for all deployment scenarios

NOTE 1:	The above NRM fragment uses SubNetwork to hold both NR and LTE external entities and frequencies.

Figure 4.2.1.1-5: Cell Relation view for all deployment scenarios

NOTE 2:	The above NRM fragment uses NRNetwork to hold NR external entities and frequency and using EUtraNetwork to hold LTE external entities and frequency. The NRNetwork and EUtraNetwork are subclasses of SubNetwork (defined in TS 28.622 [30]) with no additional attributes. The reason using NRNetwork and EUtraNetwork is for a clean separation of NR external entities and frequency and LTE external entities and frequency.

Figure 4.2.1.1-6: NRM fragment for RRM Policies

Figure 4.2.1.1-7: NRM fragment to support RIM

The Figure 4.2.1.1-8 shows the NRM fragment for pre-configured 5QIs in NG-RAN.

Figure 4.2.1.1-8: NRM fragment for pre-configured 5QIs in NG-RAN

Figure 4.2.1.1-9: NRM fragment for DANR Management

Figure 4.2.1.1-10: NRM fragment for DES Management

Figure 4.2.1.1-11: NRM fragment for DRACH Management

Figure 4.2.1.1-12: NRM fragment for DMRO Management

Figure 4.2.1.1-13: NRM fragment for DPCI Management

Figure 4.2.1.1-14: NRM fragment for CES Management

Figure 4.2.1.1-15: NRM fragment for CPCI Management

The Figure 4.2.1.1-16 shows the NRM fragment for dynamic 5QIs in NG-RAN.

Figure 4.2.1.1-16: NRM fragment for dynamically assigned 5QIs in NG-RAN

#### 4.2.1.2	Inheritance

Figure 4.2.1.2-1: Inheritance Hierarchy

Figure 4.2.1.2-2: Inheritance Hierarchy

## 4.3	Class definitions

### 4.3.1	GNBDUFunction

#### 4.3.1.1	Definition

For non-split NG-RAN deployment scenario, this IOC together with GNBCUCPFunction IOC and GNBCUUPFunction IOC provide the management of gNB defined in clause 6.1.1 in 3GPP TS 38.401 [4].

For 2-split and 3-split NG-RAN architecture, this IOC provides the management representation of gNB-DU defined in clause 6.1.1 in 3GPP TS 38.401 [4].

The following table identifies the necessary end points required for the representation of gNB and en-gNB, of all deployment scenarios.

| Req Role | End point requirement for 3-split deployment scenario | End point requirement for 2-split deployment scenario | End point requirement for Non-split deployment scenario |
| --- | --- | --- | --- |
| gNB | <<IOC>>EP_F1C, <<IOC>>EP_F1U | <<IOC>>EP_F1C, <<IOC>>EP_F1U | None. |
| en-gNB | <<IOC>>EP_F1C, <<IOC>>EP_F1U | <<IOC>>EP_F1C, <<IOC>>EP_F1U | None. |

#### 4.3.1.2	Attributes

The GNBDUFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gNBDUId | M | T | T | F | T |
| gNBDUName | O | T | T | F | T |
| gNBId | M | T | F | F | T |
| gNBIdLength | M | T | T | F | T |
| rimRSReportConf | O | T | F | T | T |

#### 4.3.1.3	Attribute constraints

None.

#### 4.3.1.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.2	GNBCUCPFunction

#### 4.3.2.1	Definition

For non-split NG-RAN deployment scenario, this IOC together with GNBCUUPFunction IOC and GNBDUFunction IOC provide the management representation of gNB defined in clause 6.1.1 in 3GPP TS 38.401 [4].

For 2-split NG-RAN deployment scenario, this IOC together with GNBCUUPFunction IOC provide management representation of the gNB-CU defined in clause 6.1.1 in 3GPP TS 38.401 [4].

For 3-split NG-RAN deployment scenario, this IOC provides management representation of gNB-CU-CP defined in clause 6.1.2 in 3GPP TS 38.401 [4].

The following table identifies the necessary end points required for the representation of gNB and en-gNB, of all deployment scenarios.

| Req Role | End point requirement for 3-split deployment scenario | End point requirement for 2-split deployment scenario | End point requirement for Non-split deployment scenario |
| --- | --- | --- | --- |
| gNB | <<IOC>>EP_XnC, <<IOC>>EP_NgC, <<IOC>>EP_F1C, <<IOC>>EP_E1. | <<IOC>>EP_XnC, <<IOC>>EP_NgC, <<IOC>>EP_F1C <<IOC>>EP_F1U. | <<IOC>>EP_XnC, <<IOC>>EP_NgC. |
| en-gNB | <<IOC>>EP_X2C, <<IOC>>EP_F1C, <<IOC>>EP_E1. | <<IOC>>EP_X2C, <<IOC>>EP_F1C. | <<IOC>>EP_X2C. |

#### 4.3.2.2	Attributes

The GNBCUCPFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gNBId | M | T | T | F | T |
| gNBIdLength | M | T | T | F | T |
| gNBCUName | O | T | T | F | T |
| pLMNId | M | T | T | T | T |
| x2BlackList | CM | T | T | F | T |
| x2WhiteList | CM | T | T | F | T |
| xnBlackList | M | T | T | F | T |
| xnWhiteList | M | T | T | F | T |
| x2XnHOBlackList | CM | T | T | F | T |
| mappingSetIDBackhaulAddressList | CM | T | T | F | T |
| tceIDMappingInfoList | CM | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| configurable5QISetRef | O | T | T | F | T |
| dynamic5QISetRef | O | T | F | F | T |

#### 4.3.2.3	Attribute constraints

| Name | Definition |
| --- | --- |
| x2BlackList | Condition: Multi-Radio Dual Connectivity with the EPC (see TS 37.340 [9] clause 4.1.2) is supported. |
| x2WhiteList | Condition: Multi-Radio Dual Connectivity with the EPC (see TS 37.340 [9] clause 4.1.2) is supported. |
| mappingSetIDBackhaulAddressList | Condition: Remote Interference Management function is supported. |
| tceIDMappingInfolist | Condition: MDT Function is supported. |

#### 4.3.2.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.3	GNBCUUPFunction

#### 4.3.3.1	Definition

For non-split NG-RAN deployment scenario, this IOC together with GNBCUCPFunction IOC and GNBDUFunction IOC provide the management representation of gNB defined in clause 6.1.1 in 3GPP TS 38.401 [4].

For 2-split NG-RAN deployment scenario, this IOC together with GNBCUCPFunction IOC provide management representation of gNB-CU defined in clause 6.1.1 in 3GPP TS 38.401 [4].

For 3-split NG-RAN deployment scenario, this IOC provides management representation of gNB-CU-UP defined in clause 6.1.2 in 3GPP TS 38.401 [4].

The following table identifies the necessary end points required for the representation of gNB and en-gNB, of all deployment scenarios.

| Req Role | End point requirement for 3-split deployment scenario | End point requirement for 2-split deployment scenario | End point requirement for Non-split deployment scenario |
| --- | --- | --- | --- |
| gNB | <<IOC>>EP_XnU, <<IOC>>EP_NgU, <<IOC>>EP_F1U, <<IOC>>EP_E1. | <<IOC>>EP_XnU, <<IOC>>EP_NgU, <<IOC>>EP_F1U. | <<IOC>>EP_XnU, <<IOC>>EP_NgU. |
| en-gNB | <<IOC>>EP_X2U, <<IOC>>EP_S1U, <<IOC>>EP_F1U, <<IOC>>EP_E1. | <<IOC>>EP_X2U, <<IOC>>EP_S1U, <<IOC>>EP_F1U. | <<IOC>>EP_X2U, <<IOC>>EP_S1U. |

#### 4.3.3.2	Attributes

The GNBCUUPFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gNBCUUPId | M | T | F | T | T |
| pLMNInfoList | M | T | T | F | T |
| gNBId | M | T | T | F | T |
| gNBIdLength | M | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| configurable5QISetRef | O | T | T | F | T |
| dynamic5QISetRef | O | T | F | F | T |

#### 4.3.3.3	Attribute constraints

None.

#### 4.3.3.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.4	NRCellCU

#### 4.3.4.1	Definition

This IOC represents the part of NR cell information that is responsible for the management of inter-cell mobility and neighbour relations via ANR.

#### 4.3.4.2	Attributes

The NRCellCU IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cellLocalId | M | T | T | F | T |
| pLMNInfoList | M | T | T (Note) | F | T |
| Attribute related to role |  |  |  |  |  |
| nRFrequencyRef | M | T | F | F | T |
| Note: Whether the attribute "pLMNId" in the PLMNInfo can be writable depends on the implementation. |  |  |  |  |  |

Note 1:	 Void.

Note 2:	Void.

#### 4.3.4.3	Void

#### 4.3.4.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.5	NRCellDU

#### 4.3.5.1	Definition

This IOC represents the part of NR cell information that describes s the specific resources instances.

An NR cell transmits SS/PBCH block and always requires downlink transmission at a certain carrier frequency with a certain channel bandwidth. Transmission may be performed from multiple sector-carriers using different transmission points, and these may be configured with different carrier frequencies and channel bandwidths, as long as they are aligned to the cell's downlink resource grids as defined in subclause 4.4 in TS 38.211 [32]. The values of arfcnDL and bSChannelBwDL attributes define the resource grids which each sector-carrier needs to be aligned to. See subclauses 5.3 and 5.4.2 of TS 38.104 for definitions of BS channel bandwidth and NR-ARFCN, respectively.

An NR cell requires an uplink in order to provide initial access. In case of TDD, the values of arfcnUL and bSChannelBwUL have to always be set to the same values as for the corresponding DL attributes. For both FDD and TDD, the arfcnUL and bSChannelBwUL define uplink resource grids to which each sector-carrier needs to align to.

An NR cell can in addition be configured with a supplementary uplink, which has its own arfcnSUL and bSChannelBwSUL, which define resource grids for supplementary uplink sector-carriers.

Each of downlink, uplink and supplementary uplink (if configured) need an initial bandwidth part (BWP), which defines resources to be used by UEs during and immediately after initial access. Additional BWPs can be either configured or calculated by gNB internally and be applied to UEs dynamically by gNB based on e.g. UE capability and bandwidth need of each UE.

NOTE: Void

#### 4.3.5.2	Attributes

The NRCellDU IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cellLocalId | M | T | T | F | T |
| operationalState | M | T | F | F | T |
| administrativeState | M | T | T | F | T |
| cellState | M | T | F | F | T |
| pLMNInfoList | M | T | T | F | T |
| nRPCI | M | T | T | F | T |
| nRTAC | CM | T | T | F | T |
| arfcnDL | M | T | T | F | T |
| arfcnUL | CM | T | T | F | T |
| arfcnSUL | CM | T | T | F | T |
| bSChannelBwDL | M | T | T | F | T |
| ssbFrequency | CM | T | T | F | T |
| ssbPeriodicity | M | T | T | F | T |
| ssbSubCarrierSpacing | CM | T | T | F | T |
| ssbOffset | M | T | T | F | T |
| ssbDuration | M | T | T | F | T |
| bSChannelBwUL | CM | T | T | F | T |
| bSChannelBwSUL | CM | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| nRSectorCarrierRef | M | T | T | F | T |
| bWPRef | M | T | T | F | T |
| nRFrequencyRef | CO | T | T | F | T |
| victimSetRef | CM | T | T | F | T |
| aggressorSetRef | O | T | T | F | T |
| Note 1: No state propagation is implied. Note 2: Void |  |  |  |  |  |

#### 4.3.5.3	Attribute constraints

| Name | Definition |
| --- | --- |
| arfcnUL Support Qualifier | Condition: The cell has an uplink (FDD or TDD) |
| arfcnSUL Support Qualifier | Condition: The cell has a supplementary uplink |
| bSChannelBwUL Support Qualifier | Condition: The cell has an uplink (FDD or TDD) |
| bSChannelBwSUL Support Qualifier | Condition: The cell has a supplementary uplink |
| nRFrequencyRef Support Qualifier | Condition: Non-split deployment scenario is supported |
| ssbFrequency Support Qualifier | Condition: nRFrequencyRef is not used. |
| ssbSubCarrierSpacing Support Qualifier | Condition: nRFrequencyRef is not used. |
| victimSetRef Support Qualifier | Condition: RIM feature is supported |

#### 4.3.5.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.6	NRSectorCarrier

#### 4.3.6.1	Definition

This <<IOC>>NRSectorCarrier represents the resources of each transmission point associated to corresponding cell(s). These in general have different physical locations (of the antennae), and possibly different frequencies or bandwidths. The UE is not directly aware of which NRSectorCarrier resources the network uses for its connection.

An NR sector-carrier can have downlink, uplink or both as specified by txDirection. Attributes related to unavailable direction (DL or UL) shall not be set.

Additional NRSectorCarriers not directly associated to one cell only can also be configured.

If a value of arfcnDL, arfcnUL, bSChannelBwDL or bSChannelBwUL can be derived unambiguously from the referring cell, then that attribute needs not be present. That will not be possible if the NRSectorCarrier is used for supplementary uplink, if it is not directly associated to a cell, or if the sector-carrier uses only a part of the cell's channel bandwidth. Thus, at least in those cases the applicable attributes have to be present and their values need to be set.

#### 4.3.6.2	Attributes

The NRSectorCarrier IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| txDirection | M | T | T | F | T |
| configuredMaxTxPower | CM | T | T | F | T |
| configuredMaxTxEIRP | CM | T | T | F | T |
| arfcnDL | CM | T | T | F | T |
| arfcnUL | CM | T | T | F | T |
| bSChannelBwDL | CM | T | T | F | T |
| bSChannelBwUL | CM | T | T | F | T |
| attribute related to role |  |  |  |  |  |
| sectorEquipmentFunctionRef | M | T | T | F | T |

#### 4.3.6.3	Attribute constraints

| Name | Definition |
| --- | --- |
| configuredMaxTxPower | Condition: The sector-carrier has a downlink. Configuration of Tx power at antenna port reference point is supported. |
| configuredMaxTxEIRP | Condition: The sector-carrier has a downlink. Configuration of emitted isotropic radiated power is supported. |
| arfcnDL | Condition: The sector-carrier has a downlink AND the value differs from the referring cell's value of arfcnDL. |
| arfcnUL | Condition: The sector-carrier has an uplink AND the value differs from the referring cell's value of arfcnUL. |
| bSChannelBwDL | Condition: The sector-carrier has a downlink AND the value differs from the referring cell's value of bSChannelBwDL. |
| bSChannelBwUL | Condition: The sector-carrier has an uplink AND the value differs from the referring cell's value of bSChannelBwUL. |

#### 4.3.6.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.7	BWP

#### 4.3.7.1	Definition

This IOC represents a bandwidth part (BWP) defined in 3GPP TS 38.211 [32], subclause 4.4.5. A bandwidth part is related to downlink, uplink or supplementary uplink resource grids, and is defined by its subcarrier spacing (SCS), cyclic prefix and location and size related to the common resource grid for the applicable SCS.

A BWP can be either an initial BWP used for initial access, or other ("regular") BWP configured for relevant UEs that support the BWP's characteristics.

#### 4.3.7.2	Attributes

The BWP IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| bwpContext | M | T | T | F | T |
| isInitialBwp | M | T | T | F | T |
| subCarrierSpacing | M | T | T | F | T |
| cyclicPrefix | M | T | T | F | T |
| startRB | M | T | T | F | T |
| numberOfRBs | M | T | T | F | T |

#### 4.3.7.3	Attribute constraints

None.

#### 4.3.7.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.8	EP_E1

#### 4.3.8.1	Definition

This IOC represents the local end point of the logical link, supporting E1 interface between gNB-CU-CP and gNB-CU-UP. The E1 interface is defined in 3GPP TS 38.401 [4].

#### 4.3.8.2	Attributes

The EP_E1 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.8.3	Attribute constraints

None.

#### 4.3.8.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.9	EP_XnU

#### 4.3.9.1	Definition

This IOC represents the one end-point of a logical link supporting the Xn user plane (Xn-U) interface. The Xn-U interface provides non-guaranteed delivery of user plane PDUs between two NG-RAN nodes. The user plane PDUs are carried on GTP-U/UDP/IP/Data link layer/Physical layer stack. See subclause 7.2 of 3GPP TS 38.420 [6].

#### 4.3.9.2	Attributes

The EP_XnU IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.9.3	Attribute constraints

None.

#### 4.3.9.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.10	EP_NgC

#### 4.3.10.1	Definition

This IOC represents the local end point of the control plane interface (NG-C) between the gNB and NG-Core entity. The transport network layer is built on IP transport. For the reliable transport of signalling messages, SCTP is added on top of IP. The application layer signalling protocol is referred to as NG-AP (NG Application Protocol).

3GPP TS 38.470 [7] noted that "one gNB-CU and a set of gNB-DUs are visible to other logical nodes as a gNB or an en-gNB where the gNB terminates the Xn and the NG interfaces, and the en-gNB terminates the X2 and the S1-U interfaces".

#### 4.3.10.2	Attributes

The EP_NgC IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.10.3	Attribute constraints

None.

#### 4.3.10.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.11	EP_NgU

#### 4.3.11.1	Definition

This IOC represents the local end point of the NG user plane (NG-U) interface between the gNB and UPF. The interface provides non-guaranteed delivery of user plane PDUs between the gNB and UPF. GTP-U is baseline for this interface.

3GPP TS 38.470 [7] noted that "one gNB-CU and a set of gNB-DUs are visible to other logical nodes as a gNB or an en-gNB where the gNB terminates the Xn and the NG interfaces, and the en-gNB terminates the X2 and the S1-U interfaces".

#### 4.3.11.2	Attributes

The EP_NgU IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| epTransportRef | O | T | F | F | T |

#### 4.3.11.3	Attribute constraints

None.

#### 4.3.11.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.12	EP_F1C

#### 4.3.12.1	Definition

This IOC represents the local end point of the control plane interface (F1-C) between the DU and CU or CU-CP. The transport network layer is based on IP transport with the SCTP on top of IP. The application layer signalling protocol is referred to as NG-AP (NG Application Protocol). See subclause 7.1 of 3GPP TS 38.470 [7].

3GPP TS 38.470 [7] noted that "one gNB-CU and a set of gNB-DUs are visible to other logical nodes as a gNB or an en-gNB where the gNB terminates the Xn and the NG interfaces, and the en-gNB terminates the X2 and the S1-U interfaces".

#### 4.3.12.2	Attributes

The EP_F1C IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.12.3	Attribute constraints

None.

#### 4.3.12.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.13	EP_F1U

#### 4.3.13.1	Definition

This IOC represents the local end point of the user plane interface (F1-U) between the DU and CU or CU-UP. The transport network layer is based on IP transport, with the UDP and GTP-U on top of IP.

3GPP TS 38.470 [7] noted that "one gNB-CU and a set of gNB-DUs are visible to other logical nodes as a gNB or an en-gNB where the gNB terminates the Xn and the NG interfaces, and the en-gNB terminates the X2 and the S1-U interfaces".

#### 4.3.13.2	Attributes

The EP_F1U IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.13.3	Attribute constraints

None.

#### 4.3.13.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.14	EP_S1U

#### 4.3.14.1	Definition

This IOC represents the local end point of the logical link, supporting S1-U interface towards a S-GW node. The S1-U interface is defined in 3GPP TS 36.410 [14].

#### 4.3.14.2	Attributes

The EP_S1U IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.14.3	Attribute constraints

None.

#### 4.3.14.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.15	EP_X2C

#### 4.3.15.1	Definition

This IOC represents the local end point of the logical link, supporting X2-C application protocols used in EN-DC, to a neighbour eNB or en-gNB node, which is defined in 3GPP TS 36.423 [15]. EN-DC is defined in 3GPP TS 37.340 [9].

#### 4.3.15.2	Attributes

The EP_X2C IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.15.3	Attribute constraints

None.

#### 4.3.15.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.16	EP_X2U

#### 4.3.16.1	Definition

This IOC represents the local end-point of a logical link supporting the X2 user plane (X2-U) interface used in EN-DC, which is defined in 3GPP TS 36.425 [16].

#### 4.3.16.2	Attributes

The EP_X2U IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.16.3	Attribute constraints

None.

#### 4.3.16.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.17	EP_XnC

#### 4.3.17.1	Definition

This IOC represents the local gNB node end point of the logical link, supporting Xn Application protocols, to a neighbour NG-RAN node (including gNB and ng-eNB). The Xn Application PDUs are carried over SCTP/IP/Data link layer/Physical layer stack. See subclause 7 of 3GPP TS 38.420 [6].

#### 4.3.17.2	Attributes

The EP_XnC IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 4.3.17.3	Attribute constraints

None

#### 4.3.17.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.18	ExternalGNBCUCPFunction

#### 4.3.18.1	Definition

This IOC represents the properties, known by the management function, of a GNBCUCPFunction managed by another management function. For more information about GNBCUCPFunction, see subclause 4.3.2.

#### 4.3.18.2	Attributes

The ExternalGNBCUCPFunction includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gNBId | M | T | T | F | T |
| gNBIdLength | M | T | T | F | T |
| pLMNId | M | T | T | F | T |

#### 4.3.18.3	Attribute constraints

None.

#### 4.3.18.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.19	ExternalGNBCUUPFunction

#### 4.3.19.1	Definition

This IOC represents the properties, known by the management function, of a GNBCUUPFunction managed by another management function. For more information about GNBCUUPFunction, see subclause 4.3.3.

#### 4.3.19.2	Attributes

The ExternalGNBCUUPFunction includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gNBId | M | T | T | F | T |
| gNBIdLength | M | T | T | F | T |

#### 4.3.19.3	Attribute constraints

None.

#### 4.3.19.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.20	ExternalGNBDUFunction

#### 4.3.20.1	Definition

This IOC represents the properties, known by the management function, of a GNBDUFunction managed by another management function. For more information about GNBDUFunction, see subclause 4.3.1.

#### 4.3.20.2	Attributes

The ExternalGNBDUFunction includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gNBId | M | T | T | T | T |
| gNBIdLength | M | T | T | F | T |

#### 4.3.20.3	Attribute constraints

None.

#### 4.3.20.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.21	ExternalUPFFunction

#### 4.3.21.1	Definition

This IOC represents the properties, known by the management function, of a UPFFunction managed by another management function. For more information about UPFFunction, see subclause 5.3.3.

#### 4.3.21.2	Attributes

The ExternalUPFFunction includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |

#### 4.3.21.3	Attribute constraints

None.

#### 4.3.21.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.22	ExternalAMFFunction

#### 4.3.22.1	Definition

This IOC represents the properties, known by the management function, of an AMFFunction managed by another management function. For more information about AMFFunction, see subclause 5.3.

#### 4.3.22.2	Attributes

The ExternalAMFFunction includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |

#### 4.3.22.3	Attribute constraints

None

#### 4.3.22.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.23	Void

### 4.3.24	ENBFunction <<ProxyClass>>

#### 4.3.24.1	Definition

This IOC represents an <<IOC>>ENBFunction and <<IOC>>ExternalENBFunction.

#### 4.3.24.2	Attributes

See that defined in <<IOC>>ENBFunction and <<IOC>>ExternalENBFunction.

#### 4.3.24.3	Attribute constraints

See that defined in <<IOC>>ENBFunction and <<IOC>>ExternalENBFunction.

#### 4.3.24.4	Notifications

See respective IOCs.

### 4.3.25	GNBCUCPFunction <<ProxyClass>>

#### 4.3.25.1	Definition

This IOC represents an <<IOC>>GNBCUCPFunction and <<IOC>>ExternalGNBCUCPFunction.

#### 4.3.25.2	Attributes

See that defined in <<IOC>>GNBCUCPFunction and <<IOC>>ExternalGNBCUCPFunction.

#### 4.3.25.3	Attribute constraints

See respective IOCs.

#### 4.3.25.4	Notifications

See respective IOCs.

### 4.3.26	GNBCUUPFunction <<ProxyClass>>

#### 4.3.26.1	Definition

This IOC represents an <<IOC>>GNBCUUPFunction and <<IOC>>ExternalGNBCUUPFunction.

#### 4.3.26.2	Attributes

See that defined in <<IOC>>GNBCUUPFunction and <<IOC>>ExternalGNBCUUPFunction.

#### 4.3.26.3	Attribute constraints

See that defined in <<IOC>>GNBCUUPFunction and <<IOC>>ExternalGNBCUUPFunction.

#### 4.3.26.4	Notifications

See respective IOCs.

### 4.3.27	GNBDUFunction <<ProxyClass>>

#### 4.3.27.1	Definition

This IOC represents an <<IOC>>GNBDUFunction and <<IOC>>ExternalGNBDUFunction.

#### 4.3.27.2	Attributes

See that defined in <<IOC>>GNBDUFunction and <<IOC>>ExternalGNBDUFunction.

#### 4.3.27.3	Attribute constraints

See that defined in <<IOC>>GNBDUFunction and <<IOC>>ExternalGNBDUFunction.

#### 4.3.27.4	Notifications

See respective IOCs.

### 4.3.28	ServingGWFFunction <<ProxyClass>>

#### 4.3.28.1	Definition

This IOC represents an <<IOC>>ServingGWFFunction and <<IOC>>ExternalServingGWFunction.

#### 4.3.28.2	Attributes

See that defined in <<IOC>>ServingGWFunction and <<IOC>>ExternalServingGWFunction.

#### 4.3.28.3	Attribute constraints

See that defined in <<IOC>>ServingGWFunction and <<IOC>>ExternalServingGWFunction.

#### 4.3.28.4	Notifications

See respective IOCs.

### 4.3.29	UPFFunction <<ProxyClass>>

#### 4.3.29.1	Definition

This IOC represents an <<IOC>>UPFFunction and <<IOC>>ExternalUPFFunction.

#### 4.3.29.2	Attributes

See that defined in <<IOC>>UPFFunction and <<IOC>>ExternalUPFFunction.

#### 4.3.29.3	Attribute constraints

See that defined in <<IOC>>UPFFunction and <<IOC>>ExternalUPFFunction.

#### 4.3.29.4	Notifications

See respective IOCs.

### 4.3.30	AMFFunction <<ProxyClass>>

#### 4.3.30.1	Definition

This IOC represents an <<IOC>>AMFFunction and <<IOC>>ExternalAMFFunction.

#### 4.3.30.2	Attributes

See that defined in <<IOC>>AMFFunction and <<IOC>>ExternalAMFFunction.

#### 4.3.30.3	Attribute constraints

See that defined in <<IOC>>AMFFunction and <<IOC>>ExternalAMFFunction.

#### 4.3.30.4	Notifications

See respective IOCs.

### 4.3.31	Void

### 4.3.32	NRCellRelation

#### 4.3.32.1	Definition

This IOC represents a neighbour cell relation from a source cell to a target cell, where the target cell is an NRCellCU or ExternalNRCellCU instance.

The source cell can be a NRCellCU instance. This is the case for an Intra-NR neighbour cell relation.

The source cell can be a EUtranGenericCell instance. This is the case for Inter-LTE-NR neighbour cell relation, from E-UTRAN to NR. See 3GPP TS 28.658 [19].

Neighbour cell relations are unidirectional.

#### 4.3.32.2	Attributes

The NRCellRelation IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name |  | Support Qualifier |  | isReadable |  | isWritable |  | isInvariant |  | isNotifyable |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| nRTCI |  | O |  | T |  | T |  | F |  | T |  |
| cellIndividualOffset |  | M |  | T |  | T |  | F |  | T |  |
| isRemoveAllowed |  | CM |  | T |  | T |  | F |  | T |  |
| isHOAllowed |  | CM |  | T |  | T |  | F |  | T |  |
| isESCoveredBy |  | CM |  | T |  | T |  | F |  | T |  |
| isENDCAllowed |  | CM |  | T |  | T |  | F |  | T |  |
| isMLBAllowed |  | CM |  | T |  | T |  | F |  | T |  |
| attribute related to role |  |  |  |  |  |  |  |  |  |  |  |
| nRFreqRelationRef |  | M |  | T |  | T |  | F |  | T |  |
| adjacentNRCellRef |  | M |  | T |  | T |  | F |  | T |  |

#### 4.3.32.3	Attribute constraints

| Name | Definition |
| --- | --- |
| isRemoveAllowed | Condition: ANR function is supported in the source cell. |
| isHOAllowed | Condition: ANR function is supported in the source cell. |
| isESCoveredBy | Condition: Energy Saving function is supported. |
| isENDCAllowed | Condition: Multi-Radio Dual Connectivity with the EPC (see TS 37.340 [9] clause 4.1.2) is supported. |
| isMLBAllowed | Condition: MLB function is supported in the source cell. |

#### 4.3.32.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.33	NRFreqRelation

#### 4.3.33.1	Definition

This IOC, together with the target NRFrequency, represents the frequency properties applicable to the referencing NRCellRelation.

#### 4.3.33.2	Attributes

The NRFreqRelation IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| offsetMO | O | T | T | F | F |
| blackListEntry | O | T | T | F | F |
| blackListEntryIdleMode | O | T | T | F | F |
| cellReselectionPriority | O | T | T | F | F |
| cellReselectionSubPriority | O | T | T | F | F |
| pMax | O | T | T | F | F |
| qOffsetFreq | O | T | T | F | F |
| qQualMin | O | T | T | F | F |
| qRxLevMin | M | T | T | F | F |
| threshXHighP | M | T | T | F | F |
| threshXHighQ | CM | T | T | F | F |
| threshXLowP | M | T | T | F | F |
| threshXLowQ | CM | T | T | F | F |
| tReselectionNr | M | T | T | F | F |
| tReselectionNRSfHigh | O | T | T | F | F |
| tReselectionNRSfMedium | O | T | T | F | F |
| attribute related to role |  |  |  |  |  |
| nRFrequencyRef | M | T | T | F | F |

#### 4.3.33.3	Attribute constraints

| Name | Definition |
| --- | --- |
| threshXHighQ | Condition: RSRQ used in SIB4. |
| threshXLowQ | Condition: RSRQ used in SIB4. |

#### 4.3.33.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.34	Void

### 4.3.35	ExternalNRCellCU

#### 4.3.35.1	Definition

This abstract IOC represents the properties of an NRCellCU controlled by another Management Service Provider. This IOC contains necessary attributes for inter-system and intra-system handover. It also contains a subset of the attributes of related IOCs controlled by Management Service Provider. The way to maintain consistency between the attribute values of these IOCs is outside the scope of the present document.

#### 4.3.35.2	Attributes

The ExternalNRCellCU IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cellLocalId | M | T | T | F | T |
| nRPCI | M | T | T | F | T |
| plmnIdList | M | T | T | F | T |
| attribute related to role |  |  |  |  |  |
| nRFrequencyRef | M | T | T | F | T |

#### 4.3.35.3	Attribute constraints

None.

#### 4.3.35.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.36	RRMPolicyRatio

#### 4.3.36.1	Definition

This IOC represents the properties of RRMPolicyRatio. RRMPolicyRatio is one realization of abstract RRMPolicy_ IOC. RRMPolicyRatio has three attributes, apart from those inherited (DN, resourceType, rRMPolicyMemberList).

Figure 4.3.36-1 Structure of RRMPolicyRatio

-	The attribute rRMPolicyMaxRatio defines the maximum resource usage quota for the associated rRMPolicyMemberList, including at least one of shared resources, prioritized resources and dedicated resources. The sum of the ‘rRMPolicyMaxRatio’ values assigned to all RRMPolicyRatio(s) name-contained by same MangedEntity can be greater than 100.

-	The attribute rRMPolicyMinRatio defines the minimum resource usage quota for the associated RRMPolicyMemberList, including at least one of prioritized resources and dedicated resources, which means the resources quota that need to be guaranteed for use by the associated rRMPolicyMemberList. The sum of the ‘rRMPolicyMinRatio’ values assigned to all RRMPolicyRatio(s) name-contained by same MangedEntity shall be less or equal 100.

-	The attribute rRMPolicyDedicatedRatio defines the dedicated resource usage quota for the RRMPolicyMemberList, including dedicated resources. The sum of the ‘rRMPolicyDedicatedRatio’ values assigned to all RRMPolicyRatio(s) name-contained by same MangedEntity shall be less or equal 100.

The following are the definition for above mentioned three resource categories:

-	Shared resources: means the resources that are shared with other rRMPolicyMemberList(s) (i.e. the rRMPolicyMemberList(s) defined in RRMPolicyRatio(s) name-contained by the same ManagedEntity). The shared resources are not guaranteed for use by the associated rRMPolicyMemberList. The shared resources quota is represented by [rRMPolicyMaxRatio-rRMPolicyMinRatio].

-	Priortized resources: means the resources are preferentially used by the associated RRMPolicyMemberList. These resources are guaranteed for use by the associated RRMPolicyMemberList when it needs to use them. When not used, these resources may be used by other rRMPolicyMemberList(s) (i.e. the rRMPolicyMemberList(s) defined in RRMPolicyRatio(s) name-contained by the same ManagedEntity). The prioritized resources quota is represented by [rRMPolicyMinRatio-rRMPolicyDedicatedRatio]

-	Dedicated resources: means the resources are dedicated for use by the associated RRMPolicyMemberList. These resources can not be shared even if the associated RRMPolicyMember does not use them. The Dedicated resources quota is represented by [rRMPolicyDedicatedRatio].

#### 4.3.36.2	Attributes

The RRMPolicyRatio IOC includes attributes inherited from RRMPolicy_ IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| rRMPolicyMaxRatio | M | T | T | F | T |
| rRMPolicyMinRatio | M | T | T | F | T |
| rRMPolicyDedicatedRatio | O | T | T | F | T |

#### 4.3.36.3	Attribute constraints

None

#### 4.3.36.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.37	S-NSSAI <<dataType>>

#### 4.3.37.1	Definition

This data type represents an S-NSSAI. An NSSAI is a set of supported S-NSSAI(s), an S-NSSAI is comprised of an SST (Slice/Service type) and an optional SD (Slice Differentiator) field, (See TS 23.003 [13]).

#### 4.3.37.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| sST | M | T | T | F | T |
| sD | O | T | T | F | T |

#### 4.3.37.3	Attribute constraints

None

#### 4.3.37.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.38	NRFrequency

#### 4.3.38.1	Definition

This IOC represents certain NR frequency properties.

#### 4.3.38.2	Attributes

The NRFrequency IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| absoluteFrequencySSB | M | T | T | F | T |
| sSBSubCarrierSpacing | M | T | T | F | T |
| multiFrequencyBandListNR | O | T | F | F | T |

#### 4.3.38.3	Attribute constraints

None.

#### 4.3.38.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.39	CommonBeamformingFunction

#### 4.3.39.1	Definition

This <<IOC>>CommonBeamformingFunction represents common beamforming functionality (eg: SSB beams) for the NRSectorCarrier.

The CommonBeamformingFunction provides capability to configure the advanced antenna for a sector carrier. The configuration capability is provided by selection of coverageShape, digitalTilt and digitalAzimuth. These attributes represent the wanted coverage area and radiation pattern on a sector carrier related to an antenna transmission point.

This configuration capability assumes the system shall handle configuration of SSB beams within the sector carrier. Individual SSB beams within a sector carrier cannot be independently configured as this depends on many conditions and constraints, for instance TDD patterns, allocations of PRACH occasions, SIB1 and mobility considerations.

The associated <<IOC>> Beam provides information beam direction and beam width for the associated SSB beams as a result of the configuration. The beams addressed in this definition are the common beams.  There may be more than one beam per CommonBeamformingFunction for the NRSectorCarrier.

#### 4.3.39.2	Attributes

The CommonBeamformingFunction IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| coverageShape | M | T | T | F | T |
| digitalTilt | M | T | T | F | T |
| digitalAzimuth | M | T | T | F | T |

#### 4.3.39.3	Attribute constraints

None.

#### 4.3.39.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.40	Beam

#### 4.3.40.1	Definition

This <<IOC>>Beam represents the per-Beam information required for, e.g. beam performance management utilizing measurements generated in the RAN. TS 38.104 [12] relates to beam transmission, TS 38.215 [55] to beam measurements, and TS 38.331 [54] to reporting of those measurements and associated beam failure Information Elements, clauses 5.5.3, 5.5.5.2, 6.3.2. 6.2.2.

Measurements on common beams may be correlated with associated spatial beam information to assist use cases like troubleshooting performance problems, or SON functions like Coverage & Capacity Optimization.

<<IOC>>Beam can have spatial attributes of horizontal/azimuth (ie: Phi φ-axis) and vertical/tilt (ie: Theta θ-axis) beam pointing direction and beam width attributes.  There may be more than one beam per CommonBeamformingFunction for an NRSectorCarrier.  Informational note, beam direction and width are characteristics—a representation—of directional energy vectors.

#### 4.3.40.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| beamIndex | M | T | F | F | T |
| beamType | O | T | F | F | T |
| beamAzimuth | CM | T | F | F | T |
| beamTilt | CM | T | F | F | T |
| beamHorizWidth | CM | T | F | F | T |
| beamVertWidth | CM | T | F | F | T |

#### 4.3.40.3	Attribute constraints

The Beam IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Name | Definition |
| --- | --- |
| beamAzimuth Support Qualifier | Condition: The beamType is "SSB-BEAM" and Supported by Equipment |
| beamTilt Support Qualifier | Condition: The beamType is "SSB-BEAM" and Supported by Equipment |
| beamHorizWidth Support Qualifier | Condition: The beamType is "SSB-BEAM" and Supported by Equipment |
| beamVertWidth Support Qualifier | Condition: The beamType is "SSB-BEAM" and Supported by Equipment |

### 4.3.41	PLMNInfo <<dataType>>

#### 4.3.41.1	Definition

This <<dataType>> represents the PLMN supported by the <<IOC>> using this <<dataType>> as one of its attributes. In case of network slicing feature is supported, this <<dateType>> also represents the S-NSSAI in the PLMN supported by the <<IOC>> using this <<dataType>> as one of its attributes.

#### 4.3.41.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNId | M | T | T | F | T |
| sNSSAI | CM | T | T | F | T |

#### 4.3.41.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAI Support Qualifier | Condition: Network slicing feature is supported. |

#### 4.3.41.4	Notifications

The <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.42	RRMPolicyMember <<dataType>>

#### 4.3.42.1	Definition

This <<dataType>> represents an RRM Policy member that will be part of a rRMPolicyMemberList. A RRMPolicyMember is defined by its pLMNId and sNSSAI (S-NSSAI). The members in a rRMPolicyMemberList is assigned a specific amount of RRM resources based on settings in RRMPolicy_.

#### 4.3.42.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNId | M | T | T | F | T |
| sNSSAI | CM | T | T | F | T |

#### 4.3.42.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAI Support Qualifier | Condition: Network slicing is supported |

#### 4.3.42.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.43	RRMPolicy_

#### 4.3.43.1	Definition

This IOC represents the properties of an abstract RRMPolicy. The RRMPolicy_ IOC needs to be subclassed to be instantiated. It defines two attributes apart from those inherited from TOP IOC, the resourceType attribute defines type of resource (PRB, RRC connected users, DRB usage etc.) and the rRMPolicyMemberList attribute defines the RRMPolicyMember(s)that is subject to this policy. An RRM resource (defined in resourceType attribute) is located in NRCellDU, NRCellCU, GNBDUFunction, GNBCUCPFunction or in GNBCUUPFunction. The RRMPolicyRatio IOC is one realization of a RRMPolicy_ IOC, see the inheritance in Figure 4.2.1.2-1. This RRM framework allows adding new policies, both standardized or as vendor specific, by inheriting from the abstract RRMPolicy_ IOC.

#### 4.3.43.2	Attributes

The RRMPolicy_ IOC have the following attributes, apart from those inherited from TOP IOC (defined in TS 28.622 [30]):

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| resourceType | M | T | T | F | T |
| rRMPolicyMemberList | M | T | T | F | T |

#### 4.3.43.3	Attribute constraints

None.

#### 4.3.43.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.44	RRMPolicyManagedEntity <<ProxyClass>>

#### 4.3.44.1	Definition

This represents an <<IOC>>NRCellCU, or an <<IOC>>NRCellDU or an <<IOC>>GNBCUUPFunction, or an <<IOC>>GNBCUCPFunction, or an <<IOC>>GNBDUFunction.

If <<IOC>>NRCellCU is used, which means that a RRMPolicy shall be applied to an RRM resource in the NRCellCU. The possible RRM resource(s) owned by NRCellCU is defined in the resourceType attribute.

If <<IOC>>NRCellDU is used, which means that a RRMPolicy shall be applied to an RRM resource in the NRCellDU. The possible RRM resource(s) owned by NRCellDU is defined in the resourceType attribute.

If <<IOC>>GNBCUUPFunction is used, which means that a RRMPolicy shall be applied to an RRM resource in the GNBCUUPFunction. The possible RRM resource(s) owned by GNBCUUPFunction is defined in the resourceType attribute.

If <<IOC>>GNBCUCPFunction is used, which means that a RRMPolicy shall be applied to an RRM resource in the GNBCUCPFunction. The possible RRM resource(s) owned by GNBCUCPFunction is defined in the resourceType attribute.

If <<IOC>>GNBDUFunction is used, which means that a RRMPolicy shall be applied to an RRM resource in the GNBDUFunction. The possible RRM resource(s) owned by GNBDUFunction is defined in the resourceType attribute.

#### 4.3.44.2	Attributes

See that defined in <<IOC>>NRCellCU, <<IOC>>NRCellDU, <<IOC>>GNBCUUPFunction, <<IOC>>GNBCUCPFunction or <<IOC>>GNBDUFunction.

#### 4.3.44.3	Attribute constraints

See that defined in <<IOC>>NRCellCU, <<IOC>>NRCellDU, <<IOC>>GNBCUUPFunction, <<IOC>>GNBCUCPFunction, or <<IOC>>GNBDUFunction.

#### 4.3.44.4	Notifications

See respective IOCs.

### 4.3.45	GNBCUCPNeighbour <<ProxyClass>>

#### 4.3.45.1	Definition

This IOC represents an <<IOC>>GNBCUCPFunction, <<IOC>>ExternalGNBCUCPFunction, <<IOC>>ENBFunction and <<IOC>>ExternalENBFunction.

#### 4.3.45.2	Attributes

See that defined in <<IOC>>GNBCUCPFunction, <<IOC>>ExternalGNBCUCPFunction, <<IOC>>ENBFunction and <<IOC>>ExternalENBFunction.

#### 4.3.45.3	Attribute constraints

See that defined in <<IOC>>GNBCUCPFunction, <<IOC>>ExternalGNBCUCPFunction, <<IOC>>ENBFunction and <<IOC>>ExternalENBFunction.

#### 4.3.45.4	Notifications

See respective IOCs.

### 4.3.46	GNBCUUPNeighbour <<ProxyClass>>

#### 4.3.46.1	Definition

This IOC represents an <<IOC>>GNBCUUPFunction, <<IOC>>ExternalGNBCUUPFunction, <<IOC>>ENBFunction and <<IOC>>ExternalENBFunction.

#### 4.3.46.2	Attributes

See that defined in <<IOC>>GNBCUUPFunction, <<IOC>>ExternalGNBCUUPFunction, <<IOC>>ENBFunction and <<IOC>>ExternalENBFunction.

#### 4.3.46.3	Attribute constraints

See that defined in <<IOC>>GNBCUUPFunction, <<IOC>>ExternalGNBCUUPFunction, <<IOC>>ENBFunction and <<IOC>>ExternalENBFunction.

#### 4.3.46.4	Notifications

See respective IOCs.

### 4.3.47	MappingSetIDBackhaulAddress  <<dataType>>

#### 4.3.47.1	Definition

This data type represents the properties describing the mapping relationship between set ID and backhaul address of gNB.

#### 4.3.47.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| setID | M | T | T | F | T |
| backhaulAddress | M | T | T | F | T |

#### 4.3.47.3	Attribute constraints

None.

#### 4.3.47.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.48	BackhaulAddress  <<dataType>>

#### 4.3.48.1	Definition

This data type represents the properties describing the backhaul address of gNB.

#### 4.3.48.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gNBId | M | T | T | F | T |
| tAI | M | T | T | F | T |

#### 4.3.48.3	Attribute constraints

None.

#### 4.3.48.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.49	TAI  <<dataType>>

#### 4.3.49.1	Definition

This data type represents the properties describing the TAI of gNB, which is used to uniquely identify a Tracking Area.

#### 4.3.49.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNId | M | T | T | F | T |
| nRTAC | M | T | T | F | T |

#### 4.3.49.3	Attribute constraints

None.

#### 4.3.49.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

4.3.50	RimRSGlobal

4.3.50.1	Definition

This IOC is used to represent global/common Remote Interference Management (RIM) Reference Signal (RS) resource allocated for the whole network. Resource for RIM-RS transmission is defined by Sequence domain resource, Time domain resource and Frequency resource. The configure parameters of the RIM RS resource are applied to all Sets of RIM RS Resource across gNBs/cells in the network.

4.3.50.2	Attributes

The RimRSGlobal IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following

attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| frequencyDomainPara | M | T | T | F | T |
| sequenceDomainPara | M | T | T | F | T |
| timeDomainPara | M | T | T | F | T |

4.3.50.3	Attribute constraints

None.

4.3.50.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.51	FrequencyDomainPara <<dataType>>

#### 4.3.51.1	Definition

This  data type defines configuration parameters of frequency domain resource to support RIM RS.

#### 4.3.51.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| rimRSSubcarrierSpacing | M | T | T | F | T |
| rIMRSBandwidth | M | T | T | F | T |
| nrofGlobalRIMRSFrequencyCandidates | M | T | T | F | T |
| rimRSCommonCarrierReferencePoint | M | T | T | F | T |
| rimRSStartingFrequencyOffsetIdList | M | T | T | F | T |

#### 4.3.51.3	Attribute constraints

None.

#### 4.3.51.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.52	SequenceDomainPara  <<dataType>>

#### 4.3.52.1	Definition

This  data type defines configuration parameters of sequence domain resource to support RIM RS.

#### 4.3.52.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nrofRIMRSSequenceCandidatesofRS1 | M | T | T | F | T |
| rimRSScrambleIdListofRS1 | M | T | T | F | T |
| nrofRIMRSSequenceCandidatesofRS2 | O | T | T | F | T |
| rimRSScrambleIdListofRS2 | O | T | T | F | T |
| enableEnoughNotEnoughIndication | M | T | T | F | T |
| RIMRSScrambleTimerMultiplier | M | T | T | F | T |
| RIMRSScrambleTimerOffset | M | T | T | F | T |

#### 4.3.52.3	Attribute constraints

None.

#### 4.3.52.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.53	TimeDomainPara  <<dataType>>

#### 4.3.53.1	Definition

This  data type defines configuration parameters of time domain resource to support RIM RS.

#### 4.3.53.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dlULSwitchingPeriod1 | M | T | T | F | T |
| symbolOffsetOfReferencePoint1 | M | T | T | F | T |
| dlULSwitchingPeriod2 | O | T | T | F | T |
| symbolOffsetOfReferencePoint2 | O | T | T | F | T |
| totalnrofSetIdofRS1 | M | T | T | F | T |
| totalnrofSetIdofRS2 | O | T | T | F | T |
| nrofConsecutiveRIMRS1 | M | T | T | F | T |
| nrofConsecutiveRIMRS2 | O | T | T | F | T |
| consecutiveRIMRS1List | M | T | T | F | T |
| consecutiveRIMRS2List | M | T | T | F | T |
| enablenearfarIndicationRS1 | O | T | T | F | T |
| enablenearfarIndicationRS2 | O | T | T | F | T |

#### 4.3.53.3	Attribute constraints

None.

#### 4.3.53.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.54	RimRSReportConf <<dataType>>

#### 4.3.54.1	Definition

This  data type defines RIM-RS reporting configuration.

#### 4.3.54.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| reportIndicator | M | T | T | F | T |
| reportInterval | M | T | T | F | T |
| nrofRIMRSReportInfo | M | T | T | F | T |
| maxPropagationDelay | O | T | T | F | T |
| RimRSReportInfoList | M | T | T | F | T |

#### 4.3.54.3	Attribute constraints

None.

#### 4.3.54.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.55	RimRSReportInfo <<dataType>>

#### 4.3.55.1	Definition

This  data type defines necessary reporting information derived from the detected RIM-RS, including

1)	The detected set ID;

2)	Propagation delay in number of OFDM symbols

3)	Functionality of the RS (RS-1 or RS-2, Enough or Not enough mitigation for RS-1).

#### 4.3.55.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| detectedSetID | M | T | T | F | T |
| propagationDelay | O | T | T | F | T |
| functionalityOfRIMRS | M | T | T | F | T |

#### 4.3.55.3	Attribute constraints

None.

#### 4.3.55.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

4.3.56	RimRSSet

4.3.56.1	Definition

This IOC is used to represent aggressor or victim Set organized by OAM. The RIM RS Resource is assigned to each Set, which is identified by triple indices set of  <Time domain index, Frequency domain index, and  Sequence index >. The triple indices set can be derived by setId attribute (See subclause 7.4.1.6 in TS 38.211 [32]).

4.3.56.2	Attributes

The RimRSSet IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following

attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| setId | M | T | T | F | T |
| setType | M | T | T | F | T |
| rimRSMonitoringStartTime | O | T | T | F | T |
| rimRSMonitoringStopTime | O | T | T | F | T |
| rimRSMonitoringWindowDuration | O | T | T | F | T |
| rimRSMonitoringWindowStartingOffset | O | T | T | F | T |
| rimRSMonitoringWindowPeriodicity | O | T | T | F | T |
| rimRSMonitoringOccasionInterval | O | T | T | F | T |
| rimRSMonitoringOccasionStartingOffset | O | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| nRCellDURef | M | T | F | F | T |

4.3.56.3	Attribute constraints

None.

4.3.56.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.57	DANRManagementFunction

#### 4.3.57.1	Definition

This IOC contains attributes to support the D-SON function of ANR Management (See clause 6.4.1.3 in TS 28.313 [57]).

#### 4.3.57.2	Attributes

The DANRManagementFunction IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| intrasystemANRManagementSwitch | M | T | T | F | T |
| intersystemANRManagementSwitch | M | T | T | F | T |

#### 4.3.57.3	Attribute constraints

None.

#### 4.3.57.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.58	DESManagementFunction

#### 4.3.58.1	Definition

This IOC represents the management capabilities of Distributed SON or Domain-Centralized SON Energy Saving (ES) functions (see clause 6.2.3.0 in TS 28.310 [71]). This is provided for Energy Saving purposes.

NOTE: in the case where multiple DESManagement MOIs exist at different levels of the containment tree, the DESManagement MOI at the lower level overrides the DESManagement MOIs at higher level(s) of the same containment tree.

#### 4.3.58.2	Attributes

The DESManagementFunction IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| desSwitch | M | T | T | F | T |
| intraRatEsActivationOriginalCellLoadParameters | CM | T | T | F | T |
| intraRatEsActivationCandidateCellsLoadParameters | CM | T | T | F | T |
| intraRatEsDeactivationCandidateCellsLoadParameters | CM | T | T | F | T |
| esNotAllowedTimePeriod | O | T | T | F | T |
| interRatEsActivationOriginalCellParameters | CM | T | T | F | T |
| interRatEsActivationCandidateCellParameters | CM | T | T | F | T |
| interRatEsDeactivationCandidateCellParameters | CM | T | T | F | T |
| energySavingState | M | T | F | F | T |
| isProbingCapable | O | T | F | F | T |

#### 4.3.58.3	Attribute constraints

| Name | Definition |
| --- | --- |
| intraRatEsActivationOriginalCellLoadParameters Support Qualifier | The condition is "Intra-RAT ESM is supported AND the cell acts as an original cell". |
| intraRatEsActivationCandidateCellsLoadParameters Support Qualifier | The condition is "Intra-RAT ESM is supported AND the cell acts as a candidate cell". |
| intraRatEsDeactivationCandidateCellsLoadParameters Support Qualifier | The condition is "Intra-RAT ESM is supported AND the cell acts as a candidate cell". |
| interRatEsActivationOriginalCellParameters CM Support Qualifier | The condition is "The cell acts as an original cell" AND inter-RAT ESM is supported. |
| interRatEsActivationCandidateCellParameters CM Support Qualifier | The condition is "The cell acts as a candidate cell" AND inter-RAT ESM is supported. |
| interRatEsDeactivationCandidateCellParameters CM Support Qualifier | The condition is "The cell acts as a candidate cell" AND inter-RAT ESM is supported. |

#### 4.3.58.4	Notification

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.59	DRACHOptimizationFunction

#### 4.3.59.1	Definition

This IOC contains attributes to support the D-SON function of RACH optimization (See clause 7.1.1 in TS 28.313 [57]).

NOTE: in the case where multiple DRACHOptimization MOIs exist at different levels of the containment tree, the DRACHOptimization MOI at the lower level overrides the DRACHOptimization MOIs at higher level(s) of the same containment tree.

#### 4.3.59.2	Attributes

The DRACHOptimizationFunction IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| ueAccProbilityDist | M | T | T | F | T |
| ueAccDelayProbilityDist | M | T | T | F | T |
| drachOptimizationControl | M | T | T | F | T |

#### 4.3.59.3	Attribute constraints

None.

#### 4.3.59.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.60	DMROFunction

#### 4.3.60.1	Definition

This IOC contains attributes to support the D-SON function of MRO (See clause 7.1.2 in TS 28.313 [57]).

NOTE: in the case where multiple DMRO MOIs exist at different levels of the containment tree, the DMRO MOI at the lower level overrides the DMRO MOIs at higher level(s) of the same containment tree.

#### 4.3.60.2	Attributes

The DMROFunction IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dmroControl | M | T | T | F | T |
| maximumDeviationHoTrigger | M | T | T | F | T |
| minimumTimeBetweenHoTriggerChange | M | T | T | F | T |
| tstoreUEcntxt | M | T | T | F | T |

#### 4.3.60.3	Attribute constraints

None.

#### 4.3.60.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions

### 4.3.61	DPCIConfigurationFunction

#### 4.3.61.1	Definition

This IOC contains attributes to support the Distributed SON or Domain-Centralized SON function of PCI configuration (See clause 7.1.3 in TS 28.313 [57]).

NOTE: in the case where multiple DPCIConfiguration MOIs exist at different levels of the containment tree, the DPCIConfiguration MOI at the lower level overrides the DPCIConfiguration MOIs at higher level(s) of the same containment tree.

#### 4.3.61.2	Attributes

The DPCIConfigControlFunction IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dPciConfigurationControl | M | T | T | F | T |
| nRPciList | M | T | T | F | T |

#### 4.3.61.3	Attribute constraints

None.

#### 4.3.61.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.62	CPCIConfigurationFunction

#### 4.3.62.1	Definition

This IOC contains attributes to support the Cross Domain-Centralized SON function of PCI configuration (See clause 7.2.1 in TS 28.313 [57]).

NOTE: in the case where multiple CPCIConfiguration MOIs exist at different levels of the containment tree, the CPCIConfiguration MOI at the lower level overrides the CPCIConfiguration MOIs at higher level(s) of the same containment tree.

#### 4.3.62.2	Attributes

The CPCIConfigurationFunction IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cPciConfigurationControl | M | T | T | F | T |
| cSonPciList | M | T | T | F | T |

#### 4.3.62.3	Attribute constraints

None.

#### 4.3.62.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions

### 4.3.63	CESManagementFunction

#### 4.3.63.1	Definition

This IOC represents the management capabilities of Cross Domain-Centralized SON Energy Saving (ES) functions. This is provided for Energy Saving purposes.

NOTE: in the case where multiple CESManagement MOIs exist at different levels of the containment tree, the CESManagement MOI at the lower level overrides the ESManagement MOIs at higher level(s) of the same containment tree.

#### 4.3.63.2	Attributes

The CESManagementFunction IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cesSwitch | M | T | T | F | T |
| energySavingControl | M | T | T | F | T |
| energySavingState | M | T | T | F | T |

#### 4.3.63.3	Attribute constraints

None.

#### 4.3.63.4	Notification

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.64	AddressWithVlan <<dataType>>

#### 4.3.64.1	Definition

This data type represents the address including IP address and VLAN Id (e.g. localAddress of EP_NgC) used for initialization of the underlying transport.

#### 4.3.64.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| iPAddress | O | T | T | F | T |
| vLANId | O | T | T | F | T |

#### 4.3.64.3	Attribute constraints

None

#### 4.3.64.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.65	TceIDMappingInfo  <<dataType>>

#### 4.3.65.1	Definition

This data type represents the properties describing the mapping relationship between TCE ID, PLMN where TCE resides and IP address of TCE.

#### 4.3.65.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| tceIPAddress | M | T | T | F | T |
| tceID | M | T | T | F | T |
| pLMNTarget | M | T | T | F | T |

#### 4.3.65.3	Attribute constraints

None.

#### 4.3.65.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

## 4.4	Attribute definitions

### 4.4.1	Attribute properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| administrativeState | It indicates the administrative state of the NRCellDU. It describes the permission to use or prohibition against using the cell, imposed through the OAM services. allowedValues: LOCKED, SHUTTING DOWN, UNLOCKED. The meaning of these values is as defined in ITU-T Recommendation X.731 [18]. See Annex A for Relation between the "Pre-operation state of the gNB-DU Cell" and administrative state relevant in case of 2-split and 3-split deployment scenarios. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: LOCKED isNullable: False |
| operationalState | It indicates the operational state of the NRCellDU instance. It describes whether the resource is installed and partially or fully operable (Enabled) or the resource is not installed or not operable (Disabled). allowedValues: ENABLED, DISABLED. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cellState | It indicates the usage state of the NRCellDU instance. It describes whether the cell is not currently in use (Idle), or currently in use but not configured to carry traffic (Inactive) or is currently in use and is configured to carry traffic (Active). The Inactive and Active definitions are in accordance with TS 38.401 [4]: "Inactive: the cell is known by both the gNB-DU and the gNB-CU. The cell shall not serve UEs; Active: the cell is known by both the gNB-DU and the gNB-CU. The cell should be able to serve UEs." "allowedValues: IDLE, INACTIVE, ACTIVE. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| arfcnDL | NR Absolute Radio Frequency Channel Number (NR-ARFCN) for downlink allowedValues: See TS 38.104 [12] subclause 5.4.2. Note that allowed values of NR-ARFCN are specified for each band in subclause 5.4.2.3. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| arfcnUL | NR Absolute Radio Frequency Channel Number (NR-ARFCN) for uplink allowedValues: See TS 38.104 [12] subclause 5.4.2. Note that allowed values of NR-ARFCN are specified for each band in subclause 5.4.2.3. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| arfcnSUL | NR Absolute Radio Frequency Channel Number (NR-ARFCN) for supplementary uplink allowedValues: See TS 38.104 [12] subclause 5.4.2. Note that allowed values of NR-ARFCN are specified for each band in subclause 5.4.2.3. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| beamAzimuth | The azimuth of a beam transmission, which means the horizontal beamforming pointing angle (beam peak direction) in the (Phi) φ-axis in 1/10th degree resolution. See subclauses 3.2 in TS 38.104 [12] and 7.3 in TS 38.901 [53] as well as TS 28.662 [11]. The pointing angle is the direction equal to the geometric centre of the half-power contour of the beam relative to the reference plane. Zero degree implies explicit antenna bearing (boresight). Positive angle implies clockwise from the antenna bearing. allowedValues: [-1800 ..1800] 0.1 degree | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Null isNullable: True |
| beamHorizWidth | The Horizontal beamWidth of a beam transmission, which means the horizontal beamforming half-power (3dB down) beamwidth in the (Phi) φ-axis in 1/10th degree resolution. See subclauses 3.2 in TS 38.104 [12] and 7.3 in TS 38.901 [53]. allowedValues: [0..3599] 0.1 degree | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Null isNullable: True |
| beamIndex | Index of the beam. For example, please see subclause 6.6.2 of TS 38.331 [54] where the ssb-Index in the rsIndexResults element of MeasResultNR is defined. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Null isNullable: True |
| beamTilt | The tilt of a beam transmission, which means the vertical beamforming pointing angle (beam peak direction) in the (Theta) θ-axis in 1/10th degree resolution. See subclauses 3.2 in TS 38.104 [12] and 7.3 in TS 38.901 [53] as well as TS 28.662 [11]. The pointing angle is the direction equal to the geometric centre of the half-power contour of the beam relative to the reference plane. Positive value implies downtilt. allowedValues: [-900..900] 0.1 degree | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Null isNullable: True |
| beamType | The type of the beam. allowedValues: "SSB-BEAM" | type: string multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: Null isNullable: True |
| beamVertWidth | The Vertical beamWidth of a beam transmission, which means the vertical beamforming half-power (3dB down) beamwidth in the (Theta) θ-axis in 1/10th degree resolution. See subclauses 3.2 in TS 38.104 [12] and 7.3 in TS 38.901 [53]. allowedValues: [0...1800] 0.1 degree | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Null isNullable: True |
| bSChannelBwDL | BS Channel BW in MHz. for downlink allowedValues: See BS Channel BW in TS 38.104 [12], subclause 5.3.​ | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| bSChannelBwUL | BS Channel BW in MHz.for uplink allowedValues: See BS Channel BW in TS 38.104 [12], subclause 5.3.​ | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| bSChannelBwSUL | BS Channel BW in MHz.for supplementary uplink allowedValues: See BS Channel BW in TS 38.104 [12], subclause 5.3.​ | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| configuredMaxTxPower | This is the maximum transmission power in milliwatts (mW) at the antenna port for all downlink channels, used simultaneously in a cell, added together. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| configuredMaxTxEIRP | This is the maximum emitted isotroptic radiated power (EIRP) in dBm for all downlink channels, used simultaneously in a cell, added together [12]. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| coverageShape | Identifies the sector carrier coverage shape described by the envelope of the contained SSB beams. The coverage shape is implementation dependent. allowedValues: 0 : 65535 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| digitalTilt | Digitally-controlled tilt through beamforming. It represents the vertical pointing direction of the antenna relative to the antenna bore sight, representing the total non-mechanical vertical tilt of the selected coverageShape. Positive value gives downwards tilt and negative value gives upwards tilt. allowedValues: [-900..900] 0.1 degree | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| digitalAzimuth | Digitally-controlled azimuth through beamforming. It represents the horizontal pointing direction of the antenna relative to the antenna bore sight, representing the total non-mechanical horizontal pan of the selected coverageShape. Positive value gives azimuth to the right and negative value gives an azimuth to the left. allowedValues: [-1800 ..1800] 0.1 degree | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cyclicPrefix | Cyclic prefix as defined in TS 38.211 [32], subclause 4.2. allowedValues: NORMAL, EXTENDED. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| localAddress | This parameter specifies the localAddress used for initialization of the underlying transport. The AddressWithVlan <dataType> is defined in clause 4.3.64. | type: AddressWithVlan multiplicity: 1 isOrdered: False isUnique: N/A defaultValue: None isNullable: False |
| AddressWithVlan.iPaddress | This parameter specifies the IP address used for initialization of the underlying transport. IP address can be an IPv4 address (See RFC 791 [37]) or an IPv6 address (See RFC 2373 [38]). | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| AddressWithVlan. vlanId | This parameter specifies the local VLAN Id (See IEEE 802.1Q [39]) used for initialization of the underlying transport. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| remoteAddress | Remote address including IP address used for initialization of the underlying transport. IP address can be an IPv4 address (See RFC 791 [37]) or an IPv6 address (See RFC 2373 [38]). | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gNBId | It identifies a gNB within a PLMN. The gNB ID is part of the NR Cell Identifier (NCI) of the gNB cells. See "gNB Identifier (gNB ID)" of subclause 8.2 of TS 38.300 [3]. See "Global gNB ID" in subclause 9.3.1.6 of TS 38.413 [5]. allowedValues: 0..4294967295 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gNBIdLength | This indicates the number of bits for encoding the gNB ID. See "Global gNB ID" in subclause 9.3.1.6 of TS 38.413 [5]. allowedValues: 22 .. 32. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gNBDUId | It uniquely identifies the DU at least within a gNB-CU. See 'gNB-DU ID' in subclause 9.3.1.9 of 3GPP TS 38.473 [8]. allowedValues: 0..236-1 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gNBCUUPId | It uniquely identifies the gNB-CU-UP at least within a gNB-CU-CP. See 'gNB-CU-UP ID' in subclause 9.3.1.15 of 3GPP TS 38.463 [48]. allowedValues: 0..236-1 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gNBCUName | It identifies the Central Entity of a NR node, see subclause 9.2.1.4 of 3GPP TS 38.473 [8]. allowedValues: Not applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gNBDUName | It identifies the Distributed Entity of a NR node, see subclause 9.2.1.5 of 3GPP TS 38.473 [8]. allowedValues: Not applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cellLocalId | It identifies a NR cell of a gNB. It, together with the gNB Identifier (using gNBId of the parent GNBCUCPFunction or GNBDUFunction or ExternalCUCPFunction), identifies a NR cell within a PLMN. This is the NR Cell Identity (NCI). See subclause 8.2 of TS 38.300 [3]. The NCI can be constructed by encoding the gNB Identifier using gNBId (of the parent GNBCUCPFunction or GNBDUFunction or ExternalCUCPFunction) and cellLocalId where the gNB Identifier field is of length specified by gNBIdLength (of the parent GNBCUCPFunction or GNBDUFunction or ExternalCUCPFunction). See "Global gNB ID" in subclause 9.3.1.6 of TS 38.413 [5]. The NR Cell Global identifier (NCGI) is constructed from the PLMN identity the cell belongs to and the NR Cell Identifier (NCI) of the cell. See relation between NCI and NCGI subclause 8.2 of TS 38.300 [3]. allowedValues: Not applicable | type: Integer multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| nRPCI | This holds the Physical Cell Identity (PCI) of the NR cell. allowedValues: See 3GPP TS 36.211 subclause 6.11 for legal values of pci. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nRTAC | This holds the identity of the common Tracking Area Code for the PLMNs. allowedValues: a) It is the TAC or Extended-TAC. b) A cell can only broadcast one TAC or Extended-TAC. See TS 36.300, subclause 10.1.7 (PLMNID and TAC relation). c) TAC is defined in subclause 19.4.2.3 of 3GPP TS 23.003 [13] and Extended-TAC is defined in subclause 9.3.1.29 of 3GPP TS 38.473 [8]. d) For a 5G SA (Stand Alone), it has a non-null value. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: NULL isNullable: True |
| GNBCUCPFunction.pLMNId | It specifies the PLMN identifier to be used as part of the global RAN node identity. allowedValues: Not applicable. | Type: PLMNId multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| GNBCUUPFunction.pLMNIdList | This is a list of PLMN identifiers. It defines from which set of PLMNs an UE must have as its serving PLMN to be allowed to use the GNB-CU-UP. allowedValues: Not applicable. | type: PLMNId multiplicity: 1..12 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| NRCellCU.pLMNInfoList | It defines which PLMNs that can be served by the NR cell, and which S-NSSAIs can be supported by the NR cell for corresponding PLMN in case of network slicing feature is supported allowedValues: Not applicable. | type: PLMNInfo multiplicity: 1..* isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| NRCellDU.pLMNInfoList | It defines which PLMNs that can be served by the NR cell, and which S-NSSAs can be supported by the NR cell for corresponding PLMN in case of network slicing feature is supported. The pLMNId of the first entry of the list is the PLMNId used to construct the nCGI for the NR cell. allowedValues: Not applicable. | type: PLMNInfo multiplicity: 1..* isOrdered: True isUnique: True defaultValue: None isNullable: False |
| ExternalNRCellCU.pLMNIdList | It defines which PLMNs that are assumed to be served by the NR Cell in another gNB-CU-CP. This list is either updated by the managed element itself (e.g. due to ANR, signalling over Xn etc) or by consumer over the standard interface. allowedValues: Not applicable. | Type: PLMNId multiplicity: 1..12 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| rRMPolicyMemberList | It represents the list of RRMPolicyMember (s) that the managed object is supporting. A RRMPolicyMember <<dataType>> include the PLMNId <<dataType>> and S-NSSAI <<dataType>>. allowedValues: N/A | type: RRMPolicyMember multiplicity: 1..* isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| resourceType | The resource type of interest for an RRM Policy. allowedValues: PRB (for NRCellDU, GNBDUFunction) RRC connected users (for NRCellCU, GNBCUCPFunction) DRB (for GNBCUUPFunction) See NOTE 2and NOTE 4 | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| sNSSAIList | It represents the list of S-NSSAI the managed object is supporting. The S-NSSAI is defined in 3GPP TS 23.003 [13]. allowedValues: See 3GPP TS 23.003 [13] | type: S-NSSAI multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| sST | This attribute specifies the Slice/Service type (SST) of the network slice. See clause 5.15.2 of 3GPP TS 23.501 [2]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| sD | This attribute specifies the Slice Differentiator (SD), which is optional information that complements the slice/service type(s) to differentiate amongst multiple Network Slices. See clause 5.15.2 of 3GPP TS 23.501 [2]. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| rRMPolicyMaxRatio | This attribute specifies the maximum percentage of radio resources that can be used by the associated rRMPolicyMemberList. The maximum percentage of radio resources include at least one of the shared resources, prioritized resources and dedicated resources. The sum of the ‘rRMPolicyMaxRatio’ values assigned to all RRMPolicyRatio(s) name-contained by same MangedEntity can be greater than 100. Default value: 100 allowedValues: 0 : 100 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: True allowedValues: N/A isNullable: False |
| rRMPolicyMinRatio | This attribute specifies the minimum percentage of radio resources that can be used by the associated rRMPolicyMemberList. The minimum percentage of radio resources including at least one of prioritized resources and dedicated resources. The sum of the ‘rRMPolicyMinRatio’ values assigned to all RRMPolicyRatio(s) name-contained by same MangedEntity shall be less or equal 100. Default value: 0 allowedValues: 0 : 100 NOTE: Void. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: True allowedValues: N/A isNullable: False |
| rRMPolicyDedicatedRatio | This attribute specifies the percentage of radio resource that dedicatedly used by the associated rRMPolicyMemberList. The sum of the ‘rRMPolicyDedicatedRatio’ values assigned to all RRMPolicyRatio(s) name-contained by same MangedEntity shall be less or equal 100. Default value: 0 allowedValues:0 : 100 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: TRUE allowedValues: N/A isNullable: False |
| subCarrierSpacing | Subcarrier spacing configuration for a BWP. See subclause 5 in TS 38.104 [12]. AllowedValues: [15, 30, 60, 120] depending on the frequency range FR1 or FR2. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| txDirection | Indicates if the transmission direction is downlink (DL), uplink (UL) or both downlink and uplink (DL and UL). allowedValues: DL, UL, DL and UL | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| bwpContext | It identifies whether the object is used for downlink, uplink or supplementary uplink. allowedValues: DL, UL, SUL | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| isInitialBwp | It identifies whether the object is used for initial or other BWP. allowedValues: INITIAL, OTHER | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| startRB | Offset in common resource blocks to common resource block 0 for the applicable subcarrier spacing for a BWP. This corresponds to N_BWP_start, see subclause 4.4.5 in TS 38.211 [32]. allowedValues: 0 to N_grid_size – 1, where N_grid_size equals the number of resource blocks for the BS channel bandwidth, given the subcarrier spacing of the BWP. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| numberOfRBs | Number of physical resource blocks for a BWP. This corresponds to N_BWP_size, see subclause 4.4.5 in TS 38.211 [32]. allowedValues: 1 to N_grid_size – startRB of the BWP. Se startRB for definition of N_grid_size. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nRTCI | This is the Target NR Cell Identifier. It consists of NR Cell Identifier (NCI) and Physical Cell Identifier of the target NR cell (nRPCI). The NRRelation.nRTCI identifies the target cell from the perspective of the NRCell, the name-containing instance of the subject NRCellCU instance. allowedValues: Not applicable. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| adjacentCellRef | This attribute contains the DN of an adjacentNRCell (NRCellCU or ExternalNRCellCU) allowedValues: Not applicable. | type: DN multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| ssbFrequency | Indicates cell defining SSB frequency domain position Frequency of the cell defining SSB transmission. The frequency provided in this attribute identifies the position of resource element RE=#0 (subcarrier #0) of resource block RB#10 of the SS block. The frequency must be positioned on the NR global frequency raster, as defined in TS 38.101-1 [42] subclause 5.4.2. and within bSChannelBwDL. allowedValues: 0..3279165 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nRFrequencyRef | This attribute contains the DN of the referenced NRFrequency. allowedValues: Not applicable. | type: DN multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| nRSectorCarrierRef | This attribute contains the DN of the referenced NRSectorCarrier. allowedValues: Not applicable. | type: DN multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| bWPRef | This attribute contains the DN of the referenced BWP. allowedValues: Not applicable. | type: DN multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| sectorEquipmentFunctionRef | This attribute contains the DN of the referenced NSectorEquipmentFunction. allowedValues: Not applicable. | type: DN multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| offsetMO | It is a list of offset values applicable to all measured cells with reference signal(s) indicated in this MeasObjectNR. See offsetMO of subclause 5.5.4 of TS 38.331 [54]. allowedValues: Not applicable. | type: QOffsetRangeList multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: N/A isNullable: False |
| cellIndividualOffset | It is a list of offset values for the neighbour cell. Used when UE is in connected mode. The unit is 1dB. It is defined for rsrpOffsetSSB, rsrqOffsetSSB, sinrOffsetSSB, rsrpOffsetCSI-RS, rsrqOffsetCSI-RS and sinrOffsetCSI-RS. See TS 38.331 [54]. allowedValues: Not applicable. | type: Integer multiplicity: 6 isOrdered: True isUnique: N/A defaultValue: 0 isNullable: False |
| blackListEntry | It specifies a list of PCI (physical cell identity) that are blacklisted in EUTRAN measurements as described in 3GPP TS 38.331 [54]. allowedValues: { 0…1007 } | type: Integer multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| blackListEntryIdleMode | It specifies a list of PCI (physical cell identity) that are blacklisted in SIB4 and SIB5. allowedValues: { 0…1007 } | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cellReselectionPriority | It is the absolute priority of the carrier frequency used by the cell reselection procedure. See CellReselectionPriority IE in TS 38.331 [54]. It corresponds to the parameter priority in 3GPP TS 38.304 [49]. Value 0 means lowest priority. The UE behaviour when no value is entered is specified in subclause 5.2.4.1 of 3GPP TS 38.304 [49]. The value must not already used by other RAT, i.e. equal priorities between RATs are not supported. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 0None isNullable: False |
| cellReselectionSubPriority | It indicates a fractional value to be added to the value of cellReselectionPriority to obtain the absolute priority of the concerned carrier frequency for E-UTRA and NR. See CellReselectionSubPriority IE in TS 38.331 [54]. allowedValues: { 0.2, 0.4, 0.6, 0.8 }. | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| pMax | It calculates the parameter Pcompensation (defined in 3GPP TS 38.304 [49]), at cell reselection to an Cell. Its unit is 1 dBm. It corresponds to parameter PEMAX in 3GPP TS 38.101-1 [42]. allowedValues: { -30..33 }. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| qOffsetFreq | It is the frequency specific offset applied when evaluating candidates for cell reselection. See TS 38.331 [49]. Its unit is 1 dB. allowedValues: { -24, -22, -20, -18, -16, -14, -12, -10, -8, -6, -5, -4, -3, -2, -1, 0, 1, 2, 3, 4, 5, 6, 8, 10, 12, 14, 16, 20, 22, 24 } | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 0 isNullable: False |
| qOffsetRangeList | It is used to indicate a cell, beam or measurement object specific offset to be applied when evaluating candidates for cell re-selection or when evaluating triggering conditions for measurement reporting. The value in dB. Value dB-24 corresponds to -24 dB, dB-22 corresponds to -22 dB and so on. This is a list of enum values representing, in sequence: rsrpOffsetSSB, rsrqOffsetSSB, sinrOffsetSSB, rsrpOffsetCSI-RS, srqOffsetCSI-RS, sinrOffsetCSI-RS. See Q-OffsetRangeList in subclause of subclause 6.3.1 of TS 38.331 [54]. allowedValues: { -24, -22, -20, -18, -16, -14, -12, -10, -8, -6, -5, -4, -3, -2, -1, 0, 1, 2, 3, 4, 5, 6, 8, 10, 12, 14, 16, 18, 20, 22, 24 } | type: ENUM multiplicity: 6 isOrdered: True isUnique: N/A defaultValue: 0 isNullable: False |
| qQualMin | It indicates the minimum required quality level in the cell (dB). See qQualMin in TS 38.304 [49]. Unit is 1 dB. Value 0 means that it is not sent and UE applies in such case the (default) value of negative infinity for Qqualmin. Sent in SIB3 or SIB5. allowedValues: { -34..-3, 0 } | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| qRxLevMin | It indicates the required minimum received Reference Symbol Received Power (RSRP) level in the (E-UTRA) frequency for cell reselection. It corresponds to Qrxlevmin defined in 3GPP TS 38.304 [49]. It is broadcast in SIB3 or SIB5, depending on whether the related frequency is intra- or inter-frequency. Its unit is 1 dBm and resolution is 2. allowedValues: { -140..-44 }. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| threshXHighP | This specifies the Srxlev threshold (in dB) used by the UE when reselecting towards a higher priority RAT/ frequency than the current serving frequency. Each frequency of NR and E-UTRAN might have a specific threshold. It corresponds to the ThreshX, HighP in 3GPP TS 38.304 [49]. Its unit is 1 dB and resolution is 2. allowedValues: { 0..62 } | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| threshXHighQ | This specifies the Squal threshold (in dB) used by the UE when reselecting towards a higher priority RAT/ frequency than the current serving frequency. Each frequency of NR and E-UTRAN might have a specific threshold. It corresponds to the ThreshX, HighQ in TS 38.304 [49]. Its unit is 1 dB. allowedValues: { 0..31 } | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| threshXLowP | This specifies the Srxlev threshold (in dB) used by the UE when reselecting towards a lower priority RAT/ frequency than the current serving frequency. Each frequency of NR might have a specific threshold. It corresponds to ThreshX,LowP in 3GPP TS 38.304 [49]. Its unit is 1 dB. Its resolution is 2. allowedValues: { 0..62 } | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| threshXLowQ | This specifies the Squal threshold (in dB) used by the UE when reselecting towards a lower priority RAT/ frequency than the current serving frequency. Each frequency of NR might have a specific threshold. It corresponds to ThreshX,Low in TS 38.304 [49]. Its unit is 1 dB. allowedValues: {0..31}. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| tReselectionNr | It is the cell reselection timer and corresponds to parameter TreselectionRAT for NR defined in 38.331 [54]. Its unit is in seconds. allowedValues: {0..7}. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| tReselectionNRSfHigh | The attribute t-ReselectionNr (a parameter TreselectionNR in TS 38.304 [49]) is multiplied with this factor if the UE is in high mobility state. It corresponds to the parameter Speed dependent ScalingFactor for TreselectionNr for medium high state in 3GPP TS 38.304 [49]. The unit is one %. Value mapping: 25 = 0.25 50 = 0.5 75 = 0.75 100 = 1.0 allowedValues: {25, 50, 75, 100}. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| tReselectionNRSfMedium | The attribute t-ReselectionNR (a parameter "TreselectionNR in TS 38.304 [49]”) is multiplied with this factor if the UE is in medium mobility state. It corresponds to the parameter Speed dependent ScalingFactor for TreselectionNr for medium mobility state in 3GPP TS 38.304 [49]. Its unit is one %. Value mapping: 25 = 0.25 50 = 0.5 75 = 0.75 100 = 1.0 allowedValues: {25, 50, 75, 100}. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| absoluteFrequencySSB | The absolute frequency applicable for a downlink NR carrier frequency associated with the SSB. allowedValues: {0.. 3279165}. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| sSBSubCarrierSpacing | This SSB is used for for synchronization. See subclause 5 in TS 38.104 [12]. Its units are in kHz. allowedValues: {15, 30, 120, 240}. Note that the allowed values of SSB used for representing data, by e.g. a BWP, are: 15, 30, 60 and 120 in units of kHz. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| multiFrequencyBandListNR | It is a list of additional frequency bands the frequency belongs to. The list is automatically set by the gNB. allowedValues: {1..256 } | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ssbPeriodicity | Indicates cell defined SSB periodicity in number of subframes (ms). The SSB periodicity in msec is used for the rate matching purpose. allowedValues: 5, 10, 20, 40, 80, 160. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ssbOffset | Indicates cell defining SSB time domain position. Defined as the offset of the measurement window, in number of subframes (ms), in which to receive SS/PBCH blocks, where allowed values depend on the ssbPeriodicity. allowedValues: ssbPeriodicity5 ms 0..4, ssbPeriodicity10 ms 0..9, ssbPeriodicity20 ms 0..19, ssbPeriodicity40 ms 0..39, ssbPeriodicity80 ms 0..79, ssbPeriodicity160 ms 0..159. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ssbDuration | Duration of the measurement window in which to receive SS/PBCH blocks. It is given in number of subframes (ms) (see 38.213 [41], subclause 4.1. allowedValues: 1, 2, 3, 4, 5. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rimRSMonitoringStartTime | This field configures the UTC time when the gNB attempts to start RIM-RS monitoring. allowedValues: containing the information same with xsd: dateTime. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rimRSMonitoringStopTime | This field configures the UTC time when the gNB stops RIM-RS monitoring. allowedValues: containing the information same with xsd: dateTime. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mappingSetIDBackhaulAddressList | The attribute specifies a list of mappingSetIDBackhaulAddress which is defined as a datatype (see clause 4.3.47). Which is used to retrieve the backhaul address of the victim set. allowedValues: Not applicable | type: MappingSetIDBackhaulAddress multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| backhaulAddress | The attribute specifies backhaulAddress which is defined as a datatype (see clause 4.3.48). allowedValues: Not applicable | type: BackhaulAddress multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| setID | This specifies the set ID of a victim Set (RIM-RS1 Set) or aggressor Set (RIM-RS2 set). (See subclause 7.4.1.6 in TS 38.211 [32]). allowedValues: The bit length of the set ID is maximum 22bit. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| tAI | Indicates the TAI (see subclause 9.3.3.11 in TS 38.413[5]), including pLMNId ID and nRTAC. allowedValues: Not applicable | type: TAI multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| isRemoveAllowed | This indicates if the subject NRCellRelation can be removed (deleted) or not. If TRUE, the subject NRCellRelation instance can be removed (deleted). If FALSE, the subject NRCellRelation instance shall not be removed (deleted) by any entity but an MnS consumer. allowedValues: TRUE,FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| isHOAllowed | This indicates if HO is allowed or prohibited. If TRUE, handover is allowed from source cell to target cell. The source cell is identified by the name-containing NRCellCU of the NRCellRelation that contains the isHOAllowed. The target cell is referenced by the NRCellRelation that contains this isHOAllowed. If FALSE, handover shall not be allowed. allowedValues: TRUE,FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| intrasystemANRManagementSwitch | This attribute determines whether the intra-system ANR function is activated or deactivated. If “TRUE”, the intra-system ANR function may add or remove intra NG-RAN Neighbour Relations, i.e. add or remove NRCellRelation instances from NRCellCU of this GNBCUCPFunction. If “FALSE”, the intra-system ANR Function must not add or remove Neighbour Relations, i.e. add or remove NRCellRelation instances from NRCellCU of this GNBCUCPFunction. allowedValues: TRUE,FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| intersystemANRManagementSwitch | This attribute determines whether the inter-system ANR function is activated or deactivated. If “TRUE”, the inter-system ANR function may add or remove inter-system Neighbour Relations, i.e. add or remove EUtranRelation instances from NRCellCU of this GNBCUCPFunction. If “FALSE”, the inter-system ANR Function must not add or remove inter-system Neighbour Relations, i.e. add or remove EUtranRelation instances from NRCellCU of this GNBCUCPFunction. allowedValues: TRUE,FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| desSwitch | This attribute determines whether the Distributed SON or Domain-Centralized SON energy saving function is enabled or disabled. allowedValues: TRUE,FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cesSwitch | This attribute determines whether the Cross Domain-Centralized SON energy saving function is enabled or disabled. allowedValues: TRUE,FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| energySavingControl | This attribute allows the Cross Domain-Centralized SON energy saving function to initiate energy saving activation or deactivation. allowedValues: toBeEnergySaving, toBeNotEnergySaving | type: enumeration multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| energySavingState | Specifies the status regarding the energy saving in the cell. If the value of energySavingControl is toBeEnergySaving, then it shall be tried to achieve the value isEnergySaving for the energySavingState. If the value of energySavingControl is toBeNotEnergySaving, then it shall be tried to achieve the value isNotEnergySaving for the energySavingState. allowedValues: isNotEnergySaving, isEnergySaving. | type: enumeration multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| intraRatEsActivationOriginalCellLoadParameters | This attributes is relevant, if the cell acts as an original cell. This attribute indicates the traffic load threshold and the time duration, which are used by distributed ES algorithms to allow a cell to enter the energySaving state. The time duration indicates how long the load needs to have been below the threshold. allowedValues: Threshold: Integer 0..100 (Percentage of PRB usage, see 3GPP TS 36.314 [13]) TimeDuration: Integer (in unit of seconds) | type: data type multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| intraRatEsActivationCandidateCellsLoadParameters | This attributes is relevant, if the cell acts as a candidate cell. This attribute indicates the traffic load threshold and the time duration, which are used by distributed ES algorithms level to allow a n ‘original’ cell to enter the energySaving state. Threshold and duration are applied to the candidate cell(s) which will provides coverage backup of an original cell when it is in the energySaving state. The threshold applies in the same way for a candidate cell, no matter for which original cell it will provide backup coverage. The time duration indicates how long the traffic in the candidate cell needs to have been below the threshold before any original cells which will be provided backup coverage by the candidate cell enters energy saving state. allowedValues: Threshold: Integer 0..100 (Percentage of PRB usage (see 3GPP TS 36.314 [13]) ) TimeDuration: Integer (in unit of seconds) | type: data type multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| intraRatEsDeactivationCandidateCellsLoadParameters | This attributes is relevant, if the cell acts as a candidate cell. This attribute indicates the traffic load threshold and the time duration which is used by distributed ES algorithms to allow a cell to leave the energySaving state. Threshold and time duration are applied to the candidate cell when it which provides coverage backup for the cell in energySaving state. The threshold applies in the same way for a candidate cell, no matter for which original cell it provides backup coverage. The time duration indicates how long the traffic in the candidate cell needs to have been above the threshold to wake up one or more original cells which have been provided backup coverage by the candidate cell. allowedValues: Threshold: Integer 0..100 (Percentage of PRB usage (see 3GPP TS 36.314 [13]) ) TimeDuration: Integer (in unit of seconds) | type: data type multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| esNotAllowedTimePeriod | This attribute can be used to prevent a cell entering energySaving state. This attribute indicates a list of time periods during which inter-RAT energy saving is not allowed. Time period is valid on the specified day and time of every week. allowedValues: The legal values are as follows: startTime and endTime: All values that indicate valid UTC time. endTime should be later than startTime. periodOfDay: structure of startTime and endTime. daysOfWeekList: list of weekday. weekday: Monday, Tuesday, … Sunday. List of time periods: {{ daysOfWeek daysOfWeekList, periodOfDay dailyPeriod}} | type: data type multiplicity: 0..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| interRatEsActivationOriginalCellParameters | This attribute is relevant, if the cell acts as an original cell. This attribute indicates the traffic load threshold and the time duration, which are used by distributed inter-RAT ES algorithms to allow an original cell to enter the energySaving state. The time duration indicates how long the traffic load (both for UL and DL) needs to have been below the threshold. In case the original cell is an EUTRAN cell, the load information refers to Composite Available Capacity Group IE (see 3GPP TS 36.413 [12] Annex B.1.5) and the following applies: Load = (100 - ‘Capacity Value’ ) * ‘Cell Capacity Class Value’, where ‘Capacity Value’ and ‘Cell Capacity Class Value’ are defined in 3GPP TS 36.423 [7]. In case the original cell is a UTRAN cell, the load information refers to Cell Load Information Group IE (see 3GPP TS 36.413 [12] Annex B.1.5) and the following applies: Load= ‘Load Value’ * ‘Cell Capacity Class Value’, where ‘Load Value’ and ‘Cell Capacity Class Value’ are defined in 3GPP TS 25.413 [19]. If the ‘Cell Capacity Class Value’ is not known, then ‘Cell Capacity Class Value’ should be set to 1 when calculating the load, and the load threshold should be set in range of 0..100. allowedValues: LoadThreshold: Integer 0..10000 TimeDuration: Integer 0..900 (in unit of seconds) | type: data type multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| interRatEsActivationCandidateCellParameters | This attribute is relevant, if the cell acts as a candidate cell. This attribute indicates the traffic load threshold and the time duration, which are used by distributed inter-RAT ES algorithms to allow an original cell to enter the energySaving state. Threshold and time duration are applied to the candidate cell(s) which will provides coverage backup of an original cell when it is in the energySaving state. The time duration indicates how long the traffic load (both for UL and DL) in the candidate cell needs to have been below the threshold before any original cells which will be provided backup coverage by the candidate cell enters energySaving state. In case the candidate cell is a UTRAN or GERAN cell, the load information refers to Cell Load Information Group IE(see 3GPP TS 36.413 [12] Annex B.1.5) and the following applies: Load= ‘Load Value’ * ‘Cell Capacity Class Value’, where ‘Load Value’ and ‘Cell Capacity Class Value’ are defined in 3GPP TS 25.413 [19] (for UTRAN) / TS 48.008 [20] (for GERAN). If the ‘Cell Capacity Class Value’ is not known, then ‘Cell Capacity Class Value’ should be set to 1 when calculating the load, and the load threshold should be set in range of 0..100. allowedValues: LoadThreshold: Integer 0..10000 TimeDuration: Integer 0..900 (in unit of seconds) | type: data type multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| interRatEsDeactivationCandidateCellParameters | This attribute is relevant, if the cell acts as a candidate cell. This attribute indicates the traffic load threshold and the time duration which is used by distributed inter-RAT ES algorithms to allow an original cell to leave the energySaving state. Threshold and time duration are applied to the candidate cell which provides coverage backup for the cell in energySaving state. The time duration indicates how long the traffic load (either for UL or DL) in the candidate cell needs to have been above the threshold to wake up one or more original cells which have been provided backup coverage by the candidate cell. For the load see the definition of interRatEsActivationCandidateCellParameters. allowedValues: LoadThreshold: Integer 0..10000 TimeDuration: Integer 0..900 (in unit of seconds) | type: data type multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| isProbingCapable | This attribute indicates whether this cell is capable of performing the ES probing procedure. During this procedure the eNB owning the cell indicates its presence to UEs for measurement purposes, but prevents idle mode UEs from camping on the cell and prevents incoming handovers to the same cell. If this parameter is absent, then probing is not done. allowedValues: yes, no | type: enumeration multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| dmroControl | This attribute determines whether the MRO function is enabled or disabled. allowedValues: TRUE,FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cSonPciList | This holds a list of physical cell identities that can be assigned to the pci attribute by gNB. The assignment algorithm is not specified. This attribute shall be supported if and only if the C-SON PCI configuration is supported. See TS 28.313, ref [57] subclause 7.1.3. allowedValues: See TS 38.211 [32] subclause 7.4.2.1 for legal values of pci. The number of pci in the list is 1 to 100X. | type: Integer multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ueAccProbilityDist | This is a list of target Access Probability (APn) for the RACH optimization function. Each instance APn of the list is the probability that the UE gets access on the RACH channel per cell within n number of preambles sent over an unspecified sampling period. This target is suitable for RACH optimization. allowedValues: Each element of the list, APn, is a pair (a, n) where a is the targetProbability (in %) and n is the number of preambles sent. The legal values for a are 25, 50, 75, 90. The legal values for n are 1 to 200. The number of elements specified is 4. The number of elements supported is vendor specific. The choice of supported values for a and n is vendor-specific. | type: data type multiplicity: 0..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| ueAccDelayProbilityDist | This is a list of target Access Delay probability (ADP) for the RACH optimization function. Each instance ADP of the list is the target time before the UE gets access on the RACH channel per cell, for the P percent of the successful RACH Access attempts with lowest access delay, over an unspecified sampling period. This target is suitable for RACH optimization. allowedValues: Each element of the list, ADp, is a pair (p, d) where p is the targetProbability (in %) and d is the access delay (in milliseconds). The legal values for p are 25, 50, 75, 90. The legal values for d are 10 to 560. The number of elements specified is 4. The number of elements supported is vendor specific. The choice of supported values for a and b is vendor-specific. | type: data type multiplicity: 0..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| drachOptimizationControl | This attribute determines whether the RACH Optimization function is enabled or disabled. allowedValues: TRUE,FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nRPciList | This holds a list of physical cell identities that can be assigned to the NR cells. This attribute shall be supported if D-SON PCI configuration or domain Centralized SON PCI configuration function is supported. See subclause 8.2.3, 8.3.1 in TS 28.313 [57]. allowedValues: See TS 38.211 [32] subclause 7.4.2 for legal values of pci. The number of pci in the list is 0 to 1007. | type: Integer multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dPciConfigurationControl | This attribute determines whether the Distributed SON or Domain-Centralized SON PCI configuration Function is enabled or disabled. allowedValues: TRUE,FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cPciConfigurationControl | This attribute determines whether the Cross Domain-Centralized SON PCI configuration function is enabled or disabled. allowedValues: TRUE,FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| maximumDeviationHoTrigger | This parameter defines the maximum allowed absolute deviation of the Handover Trigger, from the default point of operation (see clause 15.5.2.5 in TS 38.300 [3] and clause 9.2.2.61 in TS 38.423 [58]). allowedValues: -20..20 Unit: 0.5 dB | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| minimumTimeBetweenHoTriggerChange | This parameter defines the minimum allowed time interval between two Handover Trigger change performed by MRO. This is used to control the stability and convergence of the algorithm (see clause 15.5.2.5 in TS 38.300 [3]). allowedValues: 0..604800 Unit: Seconds | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| tstoreUEcntxt | The timer used for detection of too early HO, too late HO and HO to wrong cell. Corresponds to Tstore_UE_cntxt timer described in clause 15.5.2.5 in TS 38.300 [3]. This attribute is used for Mobility Robustness Optimization. allowedValues: 0..1023 Unit: 100 milliseconds | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| configurable5QISetRef | This is the DN of Configurable5QISet. The detailed definition for Configurable5QISet see clause 5.3.75. allowedValues: DN of the Configurable5QISet MOI. | type: String multiplicity: 0..1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| dynamic5QISetRef | This is the DN of Dynamic5QISet. The detailed definition for Dynamic5QISet see clause 5.3.94. allowedValues: DN of the Dynamic5QISet MOI. | type: String multiplicity: 0..1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| frequencyDomainPara | This attribute defines configuration parameters of frequency domain resource to support RIM RS. allowedValues: Not applicable. | type: FrequencyDomainPara multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| sequenceDomainPara | This attribute defines configuration parameters of sequence domain resource to support RIM RS. allowedValues: Not applicable. | type: SequenceDomainPara multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| timeDomainPara | This attribute defines configuration parameters of time domain resource to support RIM RS. allowedValues: Not applicable. | type: TimeDomainPara multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rimRSSubcarrierSpacing | It is the subcarrier spacing configuration (μ) for the RIM-RS. Subcarrier spacing Δf=2μ∙15 kHz. (see 38.211 [32], subclause 5.3.3). allowedValues: 0, 1 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rIMRSBandwidth | It is RIM RS bandwidth configuration in number of PRBs (see 38.211 [32], subclause 5.3.3). For carrier bandwidth larger than 20MHz, this attributer should be 96 if subcarrier spacing is15kHz; 48 or 96 if subcarrier spacing is 30kHz; For carrier bandwidth smaller than or equal to 20MHz, this attributer should be Minimum of {96 , bandwidth of downlink carrier in number of PRBs} if subcarrier spacing is15kHz; Minimum of {48, bandwidth of downlink carrier in number of PRBs } if subcarrier spacing is 30kHz; allowedValues: 1,2..96 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nrofGlobalRIMRSFrequencyCandidates | It is the number of candidate frequency resources in the whole network (NfRIM) (see 38.211 [32], subclause 7.4.1.6). allowedValues: 1,2,4 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rimRSStartingFrequencyOffsetIdList | It is a list of configured frequency offsets in units of resource blocks, where each element is the frequency offset relative to a configured reference point for RIM-RS. The size of the list is nrofGlobalRIMRSFrequencyCandidates and the resulting frequency resource blocks of RIM-RS corresponding to different configured frequency offset have no overlapping bandwidth. (see 38.211 [32], subclause 7.4.1.6). . allowedValues: 0..maxNrofPhysicalResourceBlocks-1 where maxNrofPhysicalResourceBlocks = 550 | type: Integer multiplicity: 1, 2, 4 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nrofRIMRSSequenceCandidatesofRS1 | It is the number of candidate sequences assigned for RIM RS-1 (NsRIM,1) (see 38.211 [32], subclause 7.4.1.6). It should be even when enableEnoughNotEnoughIndication for RS-1 is ON allowedValues: 1,2..8 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rimRSScrambleIdListofRS1 | It is list of configured scrambling identities for RIM RS-1 (see 38.211 [32], subclause 7.4.1.6). The size of the list is nrofRIMRSSequenceCandidatesofRS1. allowedValues: 0..2^10-1 | type: Integer multiplicity: 1, 2..8 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nrofRIMRSSequenceCandidatesofRS2 | It is the number of candidate sequences assigned for RIM RS-2 (see 38.211 [32], subclause 7.4.1.6). allowedValues: 1,2..8 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rimRSScrambleIdListofRS2 | It is list of configured scrambling identities for RIM RS-2 (see 38.211 [32], subclause 7.4.1.6).. The size of the list is nrofRIMRSSequenceCandidatesofRS2. allowedValues: 0..2^10-1 | type: Integer multiplicity: 1, 2..8 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| enableEnoughNotEnoughIndication | It is indication of whether “Enough” / “Not enough” indication functionality is enabled for RIM RS-1 (see 38.211 [32], subclause 7.4.1.6). If the indication is "enable", the first half of nrofRIMRSSequenceCandidatesofRS1 sequences indicates "Not enough mitigation", and the second half indicates "Enough mitigation", where, "Enough mitigation" indicates that IoT going back to certain level at victim side and/or no further interference mitigation actions are needed at aggressor side "Not enough mitigation" indicates that IoT exceeding certain level at victim side and/or further interference mitigation actions are needed at aggressor side allowedValues: "ENABLE", "DISABLE" see NOTE 8 | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: DISABLE isNullable: False |
| RIMRSScrambleTimerMultiplier | It is parameter multiplier factor Z for initialization seed (see 38.211 [32], subclause 7.4.1.6). allowedValues: 0,1,….2^31-1 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| RIMRSScrambleTimerOffset | It is parameter offset for initialization seed (see 38.211 [32], subclause 7.4.1.6). allowedValues: 0,1,….2^31-1 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dlULSwitchingPeriod1 | This attribute is used to configure the first uplink-downlink switching period (P1) for RIM RS transmission in the network, where one RIM RS is configured in one uplink-downlink switching period. (see 38.211 [32], subclause 7.4.1.6). When only one TDD-UL-DL-Pattern is configured, only dl-UL-SwitchingPeriod1 is configured, where P1 equals to the transmission periodicity of the TDD-UL-DL-Pattern. When two concatenated TDD-UL-DL-Patterns are configured, and RIM-RS resources is configured only in one of the TDD patterns, only dl-UL-SwitchingPeriod1 is configured, where P1 equals to the addition of the concatenated transmission periodicity of the two TDD-UL-DL-Patterns. When two concatenated TDD-UL-DL-Patterns are configured, and RIM-RS resources are configured in both TDD patterns, both dl-UL-SwitchingPeriod1 and dl-UL-SwitchingPeriod2 are configured, where P1 equals to the transmission periodicity of the first TDD-UL-DL-Pattern. See NOTE 6 allowedValues: MS0P5, MS0P625, MS1, MS1P25, MS2, MS2P5, MS4, MS5, MS10, MS20, if a single uplink-downlink period is configured for RIM-RS purposes; MS0P5, MS0P625, MS1, MS1P25, MS2, MS2P5, MS3, MS4, MS5, MS10, MS20, if two uplink-downlink periods are configured for RIM-RS purposes. see NOTE 9 | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| symbolOffsetOfReferencePoint1 | This attribute is used to configure the reference point in the first uplink-downlink switching period, which is the symbols offset of the reference point after the starting boundary of the first uplink-downlink switching period. It’s Configured together with dl-UL-SwitchingPeriod1 (see 38.211 [32], subclause 7.4.1.6). When only one TDD-UL-DL-Pattern is configured, the reference point configured for the first uplink-downlink switching period is the DL transmission boundary of the TDD-UL-DL-Pattern. When two concatenated TDD-UL-DL-Patterns are configured, and RIM-RS resources is configured only in one of the TDD patterns, the reference point configured for the first uplink-downlink switching period is the DL transmission boundary of the TDD-UL-DL-Pattern where the RIM-RS resource is configured. When two concatenated TDD-UL-DL-Patterns are configured, and RIM-RS resources are configured in both TDD patterns, the reference points configured for first uplink-downlink switching period is the DL transmission boundary of the first TDD-UL-DL-Pattern. allowedValues: 2, 3..20*2*maxNrofSymbols-1, where maxNrofSymbols=14 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dlULSwitchingPeriod2 | This attribute is used to configure the second uplink-downlink switching period (P2) for RIM RS transmission in the network, where one RIM RS is configured in one uplink-downlink switching period (see 38.211 [32], subclause 7.4.1.6). When two concatenated TDD-UL-DL-Patterns are configured, and RIM-RS resources are configured in both TDD patterns, both dl-UL-SwitchingPeriod1 and dl-UL-SwitchingPeriod2 are configured, where P2 equals to the transmission periodicity of the second TDD-UL-DL-Pattern, and where (P1 + P2) divides 20 ms. allowedValues: MS0P5, MS0P625, MS1, MS1P25, MS2, MS2P5, MS3, MS4, MS5, MS10, MS20 See NOTE 9 | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| symbolOffsetOfReferencePoint2 | This attribute is used to configure the reference point in the second uplink-downlink switching period, which is the symbol offset of the reference point after starting boundary of the second uplink-downlink switching period. Configured together with dl-UL-SwitchingPeriod2 (see 38.211 [32], subclause 7.4.1.6). When two concatenated TDD-UL-DL-Patterns are configured, and RIM-RS resources are configured in both TDD patterns, the reference points configured for second uplink-downlink switching period is the DL transmission boundary of the second TDD-UL-DL-Pattern. allowedValues: 2, 3..20*2*maxNrofSymbols-1, where maxNrofSymbols=14 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| totalnrofSetIdofRS1 | It is the total number of set IDs for RIM RS-1 (NsetIDRIM,1) (see 38.211 [32], subclause 7.4.1.6). allowedValues: 0,1...2^22-1 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| totalnrofSetIdofRS2 | It is the total number of set IDs for RIM RS-2 (see 38.211 [32], subclause 7.4.1.6). allowedValues: 0,1...2^22-1 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nrofConsecutiveRIMRS1 | It is number of consecutive uplink-downlink switching periods for RS-1 (R1) for repetition/near-far indication:. (see 38.211 [32], subclause 7.4.1.6). allowedValues: 1,2,4,8 see NOTE 7 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nrofConsecutiveRIMRS2 | It is number of consecutive uplink-downlink switching periods for RS-2 (R2) for repetition/near-far indication. (see 38.211 [32], subclause 7.4.1.6). allowedValues: 1,2,4,8 see NOTE 7 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| consecutiveRIMRS1List | It is used to configure the OFDM symbol position(s) of RIM RS-1 within the uplink-downlink switching period. It is a list of symbol offset of RIM RS-1 before the reference point. The size of the list is nrofConsecutiveRIMRS1 (see 38.211 [32], subclause 7.4.1.6). The resulting RIM RS-1 symbols and its reference point shall belong to the same 10ms frame. . allowedValues: 2,3..20*2*maxNrofSymbols-1, where maxNrofSymbols=14 | type: Integer multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| consecutiveRIMRS2List | It is used to configure the OFDM symbol position(s) of RIM RS-2 within the uplink-downlink switching period. It is a list of symbol offset of RIM RS-2 before the reference point. The size of the list is nrofConsecutiveRIMRS2 (see 38.211 [32], subclause 7.4.1.6). The resulting RIM RS-2 symbols and its reference point shall belong to the same 10ms frame. . allowedValues: 2,3..20*2*maxNrofSymbols-1, where maxNrofSymbols=14 | type: Integer multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| enablenearfarIndicationRS1 | It is indication of whether near-far functionality is enabled for RIM RS1. If the indication is “enable”, the first half of nrofConsecutiveRIMRS1 (R1) consecutive uplink-downlink switching period is for "Near" indication with R1/2 repetitions, the second half of R1 consecutive uplink-downlink switching period is for "Far" indication with R1/2 repetitions. allowedValues: "ENABLE", "DISABLE" | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: DISABLE isNullable: False |
| enablenearfarIndicationRS2 | It is indication of whether near-far functionality is enabled for RIM RS2. If the indication is “enable”, the first half of nrofConsecutiveRIMRS2 (R2) consecutive uplink-downlink switching period is for "Near" indication with R2/2 repetitions, the second half of R2 consecutive uplink-downlink switching period is for "Far" indication with R2/2 repetitions. allowedValues: "ENABLE", "DISABLE" | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: DISABLE isNullable: False |
| rimRSReportConf | It is used to configure gNBs to report the all necessary information derived from the detected RIM-RS to OAM. allowedValues: Not applicable | type: RimRSReportConf multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: N/A isNullable: False |
| reportIndicator | It is used to enable or disable the RS report on a gNB. If the indication is “enable”, the gNB starts to periodically report necessary information derived from the detected RIM-RS to OAM. If the indication is “disable”, the gNB stops reporting. allowedValues: ENABLE, DISABLE | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: DISABLE isNullable: False |
| reportInterval | It is used to define reporting interval of a gNB in ms. allowedValues: Not applicable | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nrofRIMRSReportInfo | It is used to define the maximum number of RIMRSReportInfo in a single report. allowedValues: Not applicable | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| maxPropagationDelay | It is used to define the maximum reported OFDM symbol number for the propagation delay of the detected RIM-RS in each RIMRSReportInfo. allowedValues: 0, 1..20*2*maxNrofSymbols-1, where maxNrofSymbols=14. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rimRSReportInfoList | It represents a list (the length of the list is nrofRIMRSReportInfo) of necessary information derived from the detected RIM-RS. allowedValues: Not applicable | type: RimRSReportInfo multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: N/A isNullable: False |
| detectedSetID | This attributer indicates the Set ID of the detected RIM-RS. allowedValues: 0,1...max{totalnrofSetIdofRS1, totalnrofSetIdofRS2}. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| propagationDelay | This attributer indicates the propagation delay of the detected RIM-RS, in number of OFDM symbol. allowedValues: 0, 1.. maxPropagationDelay. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| functionalityOfRIMRS | This attributer indicates the functionality of the detected RIM-RS. If the indication of enableEnoughNotEnoughIndication is “enable”, valid values are {RS2, RS1forEnoughMitigation, RS1forNotEnoughMitigation}; If the indication of enableEnoughNotEnoughIndication is “disable”, valid values are {RS1, RS2}. allowedValues: RS1, RS2, RS1forEnoughMitigation, RS1forNotEnoughMitigation | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rimRSMonitoringWindowDuration | This attributer configures a duration of the monitoring window in which gNB monitors the RIM-RS, in unit of Pt, where Pt is the RIM-RS transmission periodicity in units of uplink-downlink switching period (see 38.211 [32], subclause 7.4.1.6). This field is configured together with rimRSMonitoringInterval, rimRSMonitoringWindowStartingOffset, rimRSMonitoringOccasionInterval and rimRSMonitoringOccasionStartingOffset. The duration of the monitoring window is expected to be larger than or equal to M*Pt, where M is the interval between adjacent monitoring occasions within the monitoring window (configured by rimRSMonitoringInterval). The absolute duration of the monitoring window is not expected to be larger than the periodicity of the monitoring window (configured by rimRSMonitoringWindowPeriodicity). Only the earliest NT consecutive detection durations in each RIM-RS transmission periodicity (Pt) in the monitoring window are taken as valid time for monitoring potential interference, and they are consecutively monitored in the monitoring window, while the residual part of each RIM-RS transmission periodicity is not used for discovering potential interference, where, a consecutive detection duration spans P1*R1 (if only P1 is configured) or P1+P22*R1 (if both P1 and P2 are configured), where, R1 is the number of consecutive uplink-downlinkswitching periods for RS-1 (configured by nrofConsecutiveRIMRS1), P1 is the first uplink-downlinkswitching period (configured by dlULSwitchingPeriod1), P2 is the second uplink-downlink switching period (configured by dlULSwitchingPeriod2), and NT=NsetIDRIM,1NfRIMNsRIM,1if enableEnoughNotEnoughIndication is "disable"2NsetIDRIM,1NfRIMNsRIM,1if enableEnoughNotEnoughIndication is "enable" NsetIDRIM,1 is the total number of set IDs for RIM RS-1 (configured by totalnrofSetIdofRS1), NfRIM is the number of candidate frequency resources in the whole network (configured by nrofGlobalRIMRSFrequencyCandidates), and NsRIM,1 is the number of candidate sequences assigned for RIM RS-1 (configured by nrofRIMRSSequenceCandidatesofRS1). allowedValues: 1,2,..2^14 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rimRSMonitoringWindowPeriodicity | This attributer configures the periodicity of the monitoring window, in unit of hours. allowedValues: 1, 2, 3, 4, 6, 8, 12, 24 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rimRSMonitoringWindowStartingOffset | This attributer configures the start offset of the first monitoring window within one day, in unit of hours. allowedValues: 0,1,2..23 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rimRSMonitoringOccasionInterval | This attributer configures the interval between adjacent monitoring occasions (M) within the monitoring window, in unit of consecutive detection duration. M is expected to be prime to NT, where NT is given in above attribute rimRSMonitoringWindowDuration. allowedValues: 1,2..NT-1. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rimRSMonitoringOccasionStartingOffset | This attributer configures the start offset of the first monitoring occasions within the monitoring window (SM), in unit of consecutive detection duration. gNB starts monitoring potential interference from the SM-th consecutive detection duration in the first complete RIM-RS transmission periodicity (Pt) within the monitoring window. allowedValues: 0,1,2..M-1 where M is the the interval between adjacent monitoring occasions within the monitoring window (configured by rimRSMonitoringOccasionInterval) | Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| victimSetRef | This attribute contains the DN of a victim Set (RimRSSet) allowedValues: Not applicable. | type: DN multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| aggressorSetRef | This attribute contains the DN of an aggressor Set (RimRSSet) allowedValues: Not applicable. | type: DN multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| setType | The attribute specifies type of a RIM-RS Set . RIM RS1 is transmitted by victim to indicate its suffering remote interference, and RIM RS2 is transmitted by aggressor to measure if Remote Interference still exist allowedValues: RS1, RS2. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nRCellDURef | This attribute contains the DN of a NR Cell (NRCellDU) allowedValues: Not applicable. | type: DN multiplicity: * isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| isENDCAllowed | This indicates if EN-DC is allowed or prohibited. If TRUE, the target cell is allowed to be used for EN-DC. The target cell is referenced by the NRCellRelation that contains this isENDCAllowed. If FALSE, EN-DC shall not be allowed. allowedValues: TRUE,FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| x2BlackList | This is a list of GeNBIds. If the target node GeNBId is a member of the source node’s NRCellCU.x2BlackList, the source node is: 1) prohibited from sending X2 connection requests to the target node; 2) forced to tear down an established X2 connection to the target node; 3) not allowed to accept incoming X2 connection requests from the target node. The same GeNBId may appear here and in NRCellCU.x2WhiteList. In such case, the GeNBId in x2WhiteList shall be treated as if it is absent. allowedValues: See NOTE 5. | type: String multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| xnBlackList | This is a list of GgNBIds. If the target node GgNBId is a member of the source node’s NRCellCU.xnBlackList, the source node is: 1) prohibited from sending Xn connection requests to the target node; 2) forced to tear down an established Xn connection to the target node; 3) not allowed to accept incoming Xn connection requests from the target node. The same GgNBId may appear here and in NRCellCU.xnWhiteList. In such case, the GgNBId in xnWhiteList shall be treated as if it is absent. allowedValues: See NOTE 5. | type: String multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| x2WhiteList | This is a list of GeNBIds. If the target node GeNBId is a member of the source node’s NRCellCU.x2WhiteList, the source node is: 1) allowed to request the establishment of an X2 connection to the target node; 2) not allowed to initiate the tear down of an established X2 connection to the target node The same GeNBId may appear here and in NRCellCU.x2BlackList. In such case, the GeNBId here shall be treated as if it is absent. allowedValues: See NOTE 5. | type: String multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| xnWhiteList | This is a list of GgNBIds. If the target node GgNBId is a member of the source node’s NRCellCU.xnWhiteList, the source node is: 1) allowed to request the establishment of Xn connection with the target node; 2) not allowed to initiate the tear down of an established Xn connection to the target node The same GgNBId may appear here and in NRCellCU.xnBlackList. In such case, the GgNBId here shall be treated as if it is absent. allowedValues: See NOTE 5. | type: String multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| xnHOBlackList | This is a list of GgNBIds. For all the entries in NRCellCU.xnHOBlackList, the subject NRCellCU is prohibited to use the Xn interface for HOs even if an Xn interface exists to the target cell. allowedValues: See NOTE 5. | type: String multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| x2HOBlackList | This is a list of GeNBIds. For all the entries in NRCellCU.x2HOBlackList, the subject NRCellCU is prohibited to use the X2 interface for HOs even if an X2 interface exists to the target cell. allowedValues: See NOTE 5. | type: String multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| tceIDMappingInfoList | This attribute includes a list of TCE ID, PLMN where TCE resides and the corresponding TCE IP address. It is used in Logged MDT case to provide the information to the gNodeB or GNBCUCPFunction to get the corresponding TCE IP address when there is an MDT log received from the UE. allowedValues: Not applicable | type: tceIDMappingInfo multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| tceIPAddress | This attribute indicates IP address of TCE. (See subclause 4.1.1.9.2 in TS 32.422[68]) | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| tceID | This attribute indicates TCE Id. (See subclause 4.1.1.9.2 in TS 32.422[68]) | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| pLMNTarget | This attribute indicates PLMN where TCE resides. (See subclauses 4.1.1.9.2 and 4.9.2 in TS 32.422 [68]) | Type: PLMNId multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| isMLBAllowed | This indicates if mobility load balancing is allowed or prohibited from source cell to target cell. If TRUE, load balancing is allowed from source cell to target cell. The source cell is identified by the name-containing NRCellCU of the NRCellRelation that contains the isMLBAllowed. The target cell is referenced by the NRCellRelation that contains this isLBAllowed. In case of isHOAllowed is FALSE, mobility load balancing is prohibited by handover from source cell to target cell. If FALSE, load balancing shall be prohibited from source cell to target cell. allowedValues: TRUE,FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NOTE 1: Void NOTE 2: The radio resource can be signaling resources (e.g. RRC connected users) or user plane resources (e.g. PRB, DRB). Different RRM Policy maybe applied for different types of radio resource. E.g. RRMPolicyRatio is used for PRB resource. NOTE 3: Void NOTE 4: A RRM Policy can make use of the defined policy (e.g. RRMPolicyRatio) or a vendor specific RRM Policy. NOTE 5: For Global gNB Identifiers, the entries are formatted according to the pattern <mcc><mnc>-<gNBIdLength>-<gNBId>, where <mcc> is three digits, <mnc> two or three digits, <gNBIdLength> is a string containing a number n as digits, in the range 22 to 32, and <gNBId> is a string containing digits for the number 0 to 2n-1. For Global eNB Identifiers, the entries are formatted according to the pattern <mcc><mnc>-<eNBIdLength>-<eNBId>, where <mcc> is three digits, <mnc> two or three digits, <gNBIdLength> is a string containing a number m as digits, m being one of 18, 20, 21 or 22, and <eNBId> is a string containing digits for the number 0 to 2m-1. NOTE 6: The maximum number of total RIM RS sequence within 10ms is 32 regardless single or two uplink-downlink period are configured in the 10ms.. NOTE 7: 1. The maximum number of consecutive uplink-downlink switching periods for repetition/near-far-functionality is 8 (the number can be either 2, 4, or 8) with near-far functionality and with repetition. 2. The maximum number of consecutive uplink-downlink switching periods for repetition is 4 (the number can be either 1, 2, or 4) without near-far functionality and with repetition only. 3. The maximum number of consecutive uplink-downlink switching periods is 2 with near-far functionality only and without repetition. NOTE 8 (for information): “Not enough mitigation” means aggressor gNB needs to increase the interference mitigation level (i.e., further interference mitigation actions) (e.g., further reducing the DL transmission power on DL symbols at aggressor side), while “Enough mitigation” means aggressor gNB keeping the current interference mitigation level unchanged (i.e., no further interference mitigation actions) (e.g., remaining the DL transmission power on DL symbols unchanged at aggressor side). NOTE 9: Value MS0P5 corresponds to 0.5 ms, MS0P625 corresponds to 0.625 ms, MS1 corresponds to 1 ms, MS1P25 corresponds to 1.25 ms, and so on. |  |  |

## 4.5	Common notifications

### 4.5.1	Alarm notifications

This clause presents a list of notifications, defined in TS 28.532 [35], that an MnS consumer may receive. The notification header attribute objectClass/objectInstance shall capture the DN of an instance of a class defined in the present document.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyNewAlarm | M | -- |
| notifyClearedAlarm | M | -- |
| notifyAckStateChanged | M | -- |
| notifyAlarmListRebuilt | M | -- |
| notifyChangedAlarm | O | -- |
| notifyCorrelatedNotificationChanged | O | -- |
| notifyChangedAlarmGeneral | O | -- |
| notifyComments | O | -- |
| notifyPotentialFaultyAlarmList | O | -- |

### 4.5.2	Configuration notifications

This clause presents a list of notifications, defined in TS 28.532 [35], that an MnS consumer may receive. The notification header attribute objectClass/objectInstance shall capture the DN of an instance of a class defined in the present document.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyMOICreation | O | -- |
| notifyMOIDeletion | O | -- |
| notifyMOIAttributeValueChanges | O | -- |
| notifyEvent | O | -- |

### 4.5.3	Threshold Crossing notifications

This clause presents a list of notifications, defined in TS 28.532 [35], that an MnS consumer may receive. The notification header attribute objectClass/objectInstance shall capture the DN of an instance of a class defined in the present document.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyThresholdCrossing | M |  |

# 5	Information Model definitions for 5GC NRM

## 5.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.622 [30], IOC, SubNetwork | SubNetwork |
| TS 28.622 [30], IOC, ManagedElement | ManagedElement |
| TS 28.622 [30], IOC, ManagedFunction | ManagedFunction |
| TS 28.622 [30], IOC, EP_RP | EP_RP |
| TS 28.708 [21], IOC, ServingGWFunction | ServingGWFunction |
| TS 28.702 [20], IOC, SmsIwmscFunction | SmsIwmscFunction |
| TS 28.702 [20], IOC, SmsGmscFunction | SmsGmscFunction |
| TS 28.702 [20], IOC, GmlcFunction | GmlcFunction |
| TS 28.658 [19], dataType, PLMNId | PLMNId |

## 5.2	Class diagram

### 5.2.1	Class diagram of 5GC NFs

#### 5.2.1.1	Relationships

This clause depicts the set of classes (e.g. IOCs) that encapsulates the information relevant for NRM of 5GC NFs definitions. This clause provides the overview of the relationships of relevant classes in UML. Subsequent clauses provide more detailed specification of various aspects of these classes.

The Figure 5.2.1.1-1 shows the 5GC NF NRM containment/naming relationship.

Figure 5.2.1.1-1: 5GC NRM containment/naming relationship

The Figure 5.2.1.1-2 shows the transport view of AMF NRM.

Figure 5.2.1.1-2: Transport view of AMF NRM

The Figure 5.2.1.1-3 shows the transport view of SMF NRM.

Figure 5.2.1.1-3: Transport view of SMF NRM

The Figure 5.2.1.1-4 shows the transport view of UPF NRM.

Figure 5.2.1.1-4: Transport view of UPF NRM

The Figure 5.2.1.1-5 shows the transport view of N3IWF NRM.

Figure 5.2.1.1-5: Transport view of N3IWF NRM

The Figure 5.2.1.1-6 shows the transport view of PCF NRM.

Figure 5.2.1.1-6: Transport view of PCF NRM

The Figure 5.2.1.1-7 shows the transport view of AUSF NRM.

Figure 5.2.1.1-7: Transport view of AUSF NRM

The Figure 5.2.1.1-8 shows the transport view of UDM NRM.

Figure 5.2.1.1-8: Transport view of UDM NRM

The Figure 5.2.1.1-9 shows the transport view of NRF NRM.

Figure 5.2.1.1-9: Transport view of NRF NRM

The Figure 5.2.1.1-10 shows the transport view of NSSF NRM.

Figure 5.2.1.1-10: Transport view of NSSF NRM

The Figure 5.2.1.1-11 shows the transport view of SMSF NRM.

Figure 5.2.1.1-11: Transport view of SMSF NRM

The Figure 5.2.1.1-12 shows the transport view of 5G location service related NRM.

Figure 5.2.1.1-12: Transport view of LMF NRM

The Figure 5.2.1.1-13 shows the transport view of 5G-EIR NRM.

Figure 5.2.1.1-13: Transport view of 5G-EIR NRM

The Figure 5.2.1.1-14 shows the transport view of SEPP NRM.

Figure 5.2.1.1-14: Transport view of SEPP NRM

The Figure 5.2.1.1-15 shows the NRM fragment for control of QoS monitoring per QoS flow per UE.

Figure 5.2.1.1-15: NRM fragment for control of QoS monitoring per QoS flow per UE

The Figure 5.2.1.1-16 shows the NRM fragment for control of GTP-U path QoS monitoring.

Figure 5.2.1.1-16: NRM fragment for control of GTP-U path QoS monitoring

The Figure 5.2.1.1-17 shows the NRM fragment for pre-configured 5QIs in 5GC.

Figure 5.2.1.1-17: NRM fragment for pre-configured 5QIs in 5GC

The Figure 5.2.1.1-18 shows the NRM fragment for 5QI and DSCP mapping.

Figure 5.2.1.1-18: NRM fragment for 5QI and DSCP mapping.

The Figure 5.2.1.1-19 shows the NRM fragment for predefined PCC rule.

Figure 5.2.1.1-19: NRM fragment for predefined PCC rule

The Figure 5.2.1.1-20 shows the NRM fragment for dynamically assigned 5QIs in 5GC.

Figure 5.2.1.1-20: NRM fragment for dynamically assigned 5QIs in 5GC

#### 5.2.1.2	Inheritance

This clause depicts the inheritance relationships that exist between IOCs.

Figure 5.2.1.2-1 shows the inheritance hierarchy from IOC ManagedFunction related to the 5GC NF NRM.

Figure 5.2.1.2-1: Inheritance hierarchy from IOC ManagedFunction related to the 5GC NF NRM

Figure 5.2.1.2-2 shows the inheritance hierarchy from IOC EP_RP related to 5GC NF NRM.

Figure 5.2.1.2-2: Inheritance hierarchy from IOC EP_RP related to the 5GC NF NRM

Figure 5.2.1.2-3: Inheritance hierarchy for IOC QFQoSMonitoringControl

Figure 5.2.1.2-4: Inheritance hierarchy for IOC GtpUPathQoSMonitoringControl

Figure 5.2.1.2-5: Inheritance hierarchy for IOC Configurable5QISet

Figure 5.2.1.2-6: Inheritance hierarchy for IOC FiveQiDscpMapping

Figure 5.2.1.2-7: Inheritance hierarchy for predefined PCC rule modeling

Figure 5.2.1.2-8: Inheritance hierarchy for IOC Dynamic5QISet

### 5.2.2	Class diagram of AMF Region/AMF Set

#### 5.2.2.1	Relationships

This clause depicts the set of classes (e.g. IOCs) that encapsulates the information relevant for NRM of AMF Region/AMF Set definitions. This clause provides the overview of the relationships of relevant classes in UML. Subsequent clauses provide more detailed specification of various aspects of these classes.

The Figure 5.2.2.1-1 shows the AMF Region/AMF Set NRM containment/naming relationship.

Figure 5.2.2.1-1: AMF Region/AMF Set NRM

#### 5.2.2.2	Inheritance

This clause depicts the inheritance relationships that exist between IOCs.

Figure 5.2.2.2-1 shows the inheritance hierarchy from IOC ManagedFunction related to the AMF Region/AMF Set NRM.

Figure 5.2.2.2-1: AMF Region/AMF Set Inheritance

## 5.3	Class definitions

### 5.3.1	AMFFunction

#### 5.3.1.1	Definition

This IOC represents the AMF functionality in 5GC. For more information about the AMF, see 3GPP TS 23.501 [2].

#### 5.3.1.2	Attributes

The AMFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| aMFIdentifier | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| sNSSAIList | CM | T | F | F | T |
| managedNFProfile | M | T | T | F | T |
| commModelList | M | T | T | F | T |

#### 5.3.1.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: Network slicing feature is supported. |

#### 5.3.1.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.2	SMFFunction

#### 5.3.2.1	Definition

This IOC represents the SMF function in 5GC. For more information about the SMF, see 3GPP TS 23.501 [2].

#### 5.3.2.2	Attributes

The SMFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| nRTAClist | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| commModelList | M | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| configurable5QISetRef | O | T | T | F | T |
| dynamic5QISetRef | O | T | F | F | T |

#### 5.3.2.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: Network slicing feature is supported. |

#### 5.3.2.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.3	UPFFunction

#### 5.3.3.1	Definition

This IOC represents the UPF function in 5GC. For more information about the UPF, see 3GPP TS 23.501 [2].

#### 5.3.3.2	Attributes

The UPFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| nRTAClist | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| supportedBMOList | O | T | T | F | T |

#### 5.3.3.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList CM Support Qualifier | The condition is "network slicing feature is supported". |

#### 5.3.3.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.4	N3IWFFunction

#### 5.3.4.1	Definition

This IOC represents the N3IWF function which is used to enable non-3GPP access networks connected to the 5GC. For more information about the N3IWF, see 3GPP TS 23.501 [2].

#### 5.3.4.2	Attributes

The N3IWFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| commModelList | M | T | T | F | T |

#### 5.3.4.3	Attribute constraints

None.

#### 5.3.4.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.5	PCFFunction

#### 5.3.5.1	Definition

This IOC represents the PCF function in 5GC. For more information about the PCF, see 3GPP TS 23.501 [2].

#### 5.3.5.2	Attributes

The PCFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| commModelList | M | T | T | F | T |
| supportedBMOList | O | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| configurable5QISetRef | O | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| dynamic5QISetRef | O | T | F | F | T |

#### 5.3.5.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: network slicing feature is supported. |

#### 5.3.5.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.6	AUSFFunction

#### 5.3.6.1	Definition

This IOC represents the AUSF function in 5GC. For more information about the AUSF, see 3GPP TS 23.501 [2].

#### 5.3.6.2	Attributes

The AUSFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| commModelList | M | T | T | F | T |

#### 5.3.6.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: Network slicing feature is supported. |

#### 5.3.6.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.7	UDMFunction

#### 5.3.7.1	Definition

This IOC represents the UDM function in 5GC. For more information about the UDM, see 3GPP TS 23.501 [2].

#### 5.3.7.2	Attributes

The UDMFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| commModelList | M | T | T | F | T |

#### 5.3.5.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: network slicing feature is supported. |

#### 5.3.5.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.8	UDRFunction

#### 5.3.8.1	Definition

This IOC represents the UDR function in 5GC. For more information about the UDR, see 3GPP TS 23.501 [2].

#### 5.3.8.2	Attributes

The UDRFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| managedNFProfile | M | T | T | F | T |

#### 5.3.8.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: Network slicing feature is supported. |

#### 5.3.8.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.9	UDSFFunction

#### 5.3.9.1	Definition

This IOC represents the UDSF function which can be interacted with any other 5GC NF defined in 3GPP TS 23.501 [2]. For more information about the UDSF, see 3GPP TS 23.501 [2].

#### 5.3.9.2	Attributes

The UDSFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| managedNFProfile | M | T | T | F | T |

#### 5.3.9.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: Network slicing feature is supported. |

#### 5.3.9.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.10	NRFFunction

#### 5.3.10.1	Definition

This IOC represents the NRF function in 5GC. For more information about the NRF, see 3GPP TS 23.501 [2].

#### 5.3.10.2	Attributes

The NRFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| nFProfileList | CM | T | T | F | T |
| cNSIIdList | O | T | T | F | T |

#### 5.3.10.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: network slicing feature is supported. |
| nfProfileList Support Qualifier | Condition: NF profile is registered and deregistered by management system. |
| nSIIdList Support Qualifier | Condition: Network slicing feature is supported. |

#### 5.3.10.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.11	NSSFFunction

#### 5.3.11.1	Definition

This IOC represents the NSSF function in 5GC. For more information about the NSSF, see 3GPP TS 23.501 [2].

#### 5.3.11.2	Attributes

The NSSFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| sNSSAIList | M | T | T | F | T |
| cNSIIdList | O | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| commModelList | M | T | T | F | T |

#### 5.3.11.3	Attribute constraints

None.

#### 5.3.11.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.12	AFFunction

#### 5.3.12.1	Definition

This IOC is defined for only purpose to describe the IOCs representing its interaction interface with 5GC (i.e. EP_Rx and EP_N5). It has no any attributes defined.

### 5.3.13	DNFunction

#### 5.3.13.1	Definition

This IOC is defined for only purpose to describe the IOCs representing Data Network (DN) interaction interface with 5GC (i.e. EP_N6). It has no any attributes defined.

### 5.3.14	SMSFFunction

#### 5.3.14.1	Definition

This IOC represents the SMSF function defined in 3GPP TS 23.501 [2].

#### 5.3.14.2	Attributes

The SMSFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| commModelList | M | T | T | F | T |

#### 5.3.14.3	Attribute constraints

None.

#### 5.3.14.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.15	LMFFunction

#### 5.3.15.1	Definition

This IOC represents the LMF function defined in 3GPP TS 23.501 [2].

#### 5.3.15.2	Attributes

The LMFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| commModelList | M | T | T | F | T |

#### 5.3.15.3	Attribute constraints

None.

#### 5.3.15.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.16	NGEIRFunction

#### 5.3.16.1	Definition

This IOC represents the 5G-EIR function in 5GC. For more information about the 5G-EIR, see 3GPP TS 23.501 [2].

#### 5.3.16.2	Attributes

The NGEIRFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| commModelList | M | T | T | F | T |

#### 5.3.16.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: network slicing feature is supported. |

#### 5.3.16.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.17	SEPPFunction

#### 5.3.17.1	Definition

This IOC represents the SEPP function which support message filtering and policing on inter-PLMN control plane interface. For more information about the SEPP, see 3GPP TS 23.501 [2].

#### 5.3.17.2	Attributes

The SEPPFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNId | M | T | F | T | T |
| sEPPType | M | T | F | T | T |
| sEPPId | M | T | F | T | T |
| fqdn | M | T | T | F | T |

#### 5.3.17.3	Attribute constraints

None.

#### 5.3.17.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.18	NWDAFFunction

#### 5.3.18.1	Definition

This IOC represents the NWDAF function in 5GC. For more information about the NWDAF, see 3GPP TS 23.501 [2].

#### 5.3.18.2	Attributes

The NWDAFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| sBIFQDN | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| commModelList | M | T | T | F | T |

#### 5.3.18.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: Network slicing feature is supported. |

#### 5.3.18.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.19	EP_N2

#### 5.3.19.1	Definition

This IOC represents the N2 interface between (R)AN and AMF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.19.2	Attributes

The EP_N2 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.19.3	Attribute constraints

None.

#### 5.3.19.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.20	EP_N3

#### 5.3.20.1	Definition

This IOC represents the N3 interface between (R)AN and UPF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.20.2	Attributes

The EP_N3 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| epTransportRef | O | T | F | F | T |

#### 5.3.20.3	Attribute constraints

None.

#### 5.3.20.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.21	EP_N4

#### 5.3.21.1	Definition

This IOC represents the N4 interface between SMF and UPF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.21.2	Attributes

The EP_N4 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.21.3	Attribute constraints

None.

#### 5.3.21.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.22	EP_N5

#### 5.3.22.1	Definition

This IOC represents the N5 interface between PCF and AF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.22.2	Attributes

The EP_N5 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.22.3	Attribute constraints

None.

#### 5.3.22.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.23	EP_N6

#### 5.3.23.1	Definition

This IOC represents the N6 interface between UPF and DN, which is defined in 3GPP TS 23.501 [2].

#### 5.3.23.2	Attributes

The EP_N6 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.23.3	Attribute constraints

None.

#### 5.3.23.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.24	EP_N7

#### 5.3.24.1	Definition

This IOC represents the N7 interface between SMF and PCF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.24.2	Attributes

The EP_N7 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.24.3	Attribute constraints

None.

#### 5.3.24.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.25	EP_N8

#### 5.3.25.1	Definition

This IOC represents the N8 interface between AMF and UDM, which is defined in 3GPP TS 23.501 [2].

#### 5.3.25.2	Attributes

The EP_N8 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.25.3	Attribute constraints

None.

#### 5.3.25.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.26	EP_N9

#### 5.3.26.1	Definition

This IOC represents the N7 interface between two UPFs, which is defined in 3GPP TS 23.501 [2].

#### 5.3.26.2	Attributes

The EP_N9 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.26.3	Attribute constraints

None.

#### 5.3.26.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.27	EP_N10

#### 5.3.27.1	Definition

This IOC represents the N10 interface between SMF and UDM, which is defined in 3GPP TS 23.501 [2].

#### 5.3.27.2	Attributes

The EP_N10 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.27.3	Attribute constraints

None.

#### 5.3.27.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.28	EP_N11

#### 5.3.28.1	Definition

This IOC represents the N11 interface between AMF and SMF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.28.2	Attributes

The EP_N11 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.28.3	Attribute constraints

None.

#### 5.3.28.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.29	EP_N12

#### 5.3.29.1	Definition

This IOC represents the N12 interface between AMF and AUSF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.29.2	Attributes

The EP_N12 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.29.3	Attribute constraints

None.

#### 5.3.29.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.30	EP_N13

#### 5.3.30.1	Definition

This IOC represents the N13 interface between AUSF and UDM, which is defined in 3GPP TS 23.501 [2].

#### 5.3.30.2	Attributes

The EP_N13 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.30.3	Attribute constraints

None.

#### 5.3.30.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.31	EP_N14

#### 5.3.31.1	Definition

This IOC represents the N14 interface between two AMFs, which is defined in 3GPP TS 23.501 [2].

#### 5.3.31.2	Attributes

The EP_N14 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.31.3	Attribute constraints

None.

#### 5.3.31.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.32	EP_N15

#### 5.3.32.1	Definition

This IOC represents the N15 interface between AMF and PCF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.32.2	Attributes

The EP_N15 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.32.3	Attribute constraints

None.

#### 5.3.32.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.33	EP_N16

#### 5.3.33.1	Definition

This IOC represents the N16 interface between two SMFs, which is defined in 3GPP TS 23.501 [2].

#### 5.3.33.2	Attributes

The EP_N16 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.33.3	Attribute constraints

None.

#### 5.3.33.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.34	EP_N17

#### 5.3.34.1	Definition

This IOC represents the N17 interface between AMF and 5G-EIR, which is defined in 3GPP TS 23.501 [2].

#### 5.3.34.2	Attributes

The EP_N17 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.34.3	Attribute constraints

None.

#### 5.3.34.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.35	EP_N20

#### 5.3.35.1	Definition

This IOC represents the N20 interface between AMF and SMSF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.35.2	Attributes

The EP_N20 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.35.3	Attribute constraints

None.

#### 5.3.35.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.36	EP_N21

#### 5.3.36.1	Definition

This IOC represents the N21 interface between SMSF and UDM, which is defined in 3GPP TS 23.501 [2].

#### 5.3.36.2	Attributes

The EP_N21 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.36.3	Attribute constraints

None.

#### 5.3.36.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.37	EP_N22

#### 5.3.37.1	Definition

This IOC represents the N22 interface between AMF and NSSF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.37.2	Attributes

The EP_N22 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.37.3	Attribute constraints

None.

#### 5.3.37.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.38	EP_N26

#### 5.3.38.1	Definition

This IOC represents the N26 interface between AMF and MME, which is defined in 3GPP TS 23.501 [2].

#### 5.3.38.2	Attributes

The EP_N26 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.38.3	Attribute constraints

None.

#### 5.3.38.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.39	Void

### 5.3.40	Void

### 5.3.41	EP_S5C

#### 5.3.41.1	Definition

This IOC represents the S5-C interface between SGW and SMF/PGW-C, which is defined in 3GPP TS 23.501 [2].

#### 5.3.41.2	Attributes

The EP_S5C IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.41.3	Attribute constraints

None.

#### 5.3.41.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.42	EP_S5U

#### 5.3.42.1	Definition

This IOC represents the S5-U interface between SGW and UPF/PGW-U, which is defined in 3GPP TS 23.501 [2].

#### 5.3.42.2	Attributes

The EP_S5U IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.42.3	Attribute constraints

None.

#### 5.3.42.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.43	EP_Rx

#### 5.3.43.1	Definition

This IOC represents the Rx interface between PCF and AF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.43.2	Attributes

The EP_Rx IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.43.3	Attribute constraints

None.

#### 5.3.43.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.44	EP_MAP_SMSC

#### 5.3.44.1	Definition

This IOC represents the MAP interface between SMSF and MSC-IWMSC/GMSC, which is defined in 3GPP TS 23.040 [22].

#### 5.3.44.2	Attributes

The EP_MAP_SMSC IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.44.3	Attribute constraints

None.

#### 5.3.44.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.45	EP_NLS

#### 5.3.45.1	Definition

This IOC represents the NLs interface between AMF and LMF, which is defined in 3GPP TS 23.501 [2].

#### 5.3.45.2	Attributes

The EP_NLS IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.45.3	Attribute constraints

None.

#### 5.3.45.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.46	EP_NLG

#### 5.3.46.1	Definition

This IOC represents the NLg interface between AMF and GMLC, which is defined in 3GPP TS 23.501 [2].

#### 5.3.46.2	Attributes

The EP_NLG IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.46.3	Attribute constraints

None.

#### 5.3.46.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.47	EP_N27

#### 5.3.47.1	Definition

This IOC represents an end point of N27 interface between vNRF and hNRF, which is defined in 3GPP TS 29.510 [10].

#### 5.3.47.2	Attributes

The EP_N27 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.47.3	Attribute constraints

None.

#### 5.3.47.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.48	EP_N31

#### 5.3.48.1	Definition

This IOC represents an end point of N31 interface between vNSSF and hNSSF, which is defined in 3GPP TS 29.531 [11].

#### 5.3.48.2	Attributes

The EP_N31 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

#### 5.3.48.3	Attribute constraints

None.

#### 5.3.48.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.49	ExternalNRFFunction

#### 5.3.49.1	Definition

This IOC represents external NRF function controlled by another management domain. For more information about the NRF, see 3GPP TS 23.501 [2].

#### 5.3.49.2	Attributes

The ExternalNRFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| id | M | T | F | F | T |
| pLMNIdList | M | T | T | F | T |

#### 5.3.49.3	Attribute constraints

None.

#### 5.3.49.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.50	ExternalNSSFFunction

#### 5.3.50.1	Definition

This IOC represents external NSSF function controlled by another management domain. For more information about the NSSF, see 3GPP TS 23.501 [2].

#### 5.3.50.2	Attributes

The ExternalNSSFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| id | M | T | F | F | T |
| pLMNIdList | M | T | T | F | T |

#### 5.3.50.3	Attribute constraints

None.

#### 5.3.50.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.51	AMFSet

#### 5.3.51.1	Definition

This IOC represents the AMF Set which consists of some AMFs that serve a given area and Network Slice. For more information about the AMF Set, see 3GPP TS 23.501 [2].

#### 5.3.51.2	Attributes

The AMFSet IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| nRTAClist | M | T | T | F | T |
| aMFSetId | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| aMFRegion | M | T | T | F | T |
| aMFSetMemberList | M | T | T | F | T |

#### 5.3.51.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: Network slicing feature is supported. |

#### 5.3.51.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.52	AMFRegion

#### 5.3.52.1	Definition

This IOC represents the AMF Region which consists one or multiple AMF Sets. For more information about the AMF Region, see 3GPP TS 23.501 [2].

#### 5.3.52.2	Attributes

The AMFRegion IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| nRTAClist | M | T | T | F | T |
| aMFRegionId | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| aMFSet | M | T | T | F | T |

#### 5.3.52.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: Network slicing feature is supported. |

#### 5.3.52.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.53	ExternalAMFFunction

#### 5.3.53.1	Definition

This IOC represents an external AMF functionality used in EN-DC. For more information about the AMF, see 3GPP TS 23.501 [2].

#### 5.3.53.2	Attributes

The ExternalAMFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| Id | M | T | F | F | T |
| pLMNIdList | M | T | T | F | T |
| aMFIdentifier | M | T | T | F | T |

#### 5.3.53.3	Attribute constraints

None.

#### 5.3.53.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.54	ManagedNFProfile <<dataType>>

#### 5.3.54.1	Definition

This data type represents a Profile definition of a Managed NF (See TS 23.501 [22]).

#### 5.3.54.2	Attributes

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nfInstanceID | M | T | F | T | F |
| nfType | M | T | F | F | F |
| hostAddr | M | T | T | F | T |
| authzInfo | O | T | T | F | T |
| location | O | T | T | F | T |
| capacity | O | T | T | F | T |
| nFInfo | M | T | T | F | T |

#### 5.3.54.3	Attribute constraints

None.

#### 5.3.54.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.55	HostAddr <<choice>>

#### 5.3.55.1	Definition

This <<choice>> stereotype represents one of a set of data types as shown in Figure 5.3.55.1-1: HostAddr <<choice>> for data types.

Figure 5.3.55.1-1: HostAddr <<choice>> for data types

NOTE: The IpAddress can be IPv4 address (See RFC 791 [24]) or IPv6 address (See RFC 2373 [25]). Refer TS 23.003 [5] for Fqdn.

### 5.3.56	NFInfo <<choice>>

#### 5.3.56.1	Definition

This <<choice>> stereotype represents one of a set of data types as shown in Figure 5.3.56.1-1: NFInfo <<choice>> for data types.

Figure 5.3.56.1-1: NFInfo choice for data types

NOTE: The AmfInfo <<dataType>> is chosed for AFMFunction, the UdrInfo <<dataType>> is chosed for UDRFunction, the UdmInfo <<dataType>> is chosed for UDMFunction, the AusfInfo<<dataType>> is chosed for AUSFFunction, the UpfInfo <<dataType>> is chosed for UPFFunction

### 5.3.57	UdmInfo <<dataType>>

#### 5.3.57.1	Definition

This data type represents a generic NFProfile definition (See TS 23.501 [22]).

#### 5.3.57.2	Attributes

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nFSrvGroupId | M | T | F | F | F |

#### 5.3.57.3	Attribute constraints

None

#### 5.3.57.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.58	AusfInfo <<dataType>>

#### 5.3.58.1	Definition

This data type represents a generic NFProfile definition (See TS 23.501 [22]).

#### 5.3.58.2	Attributes

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nFSrvGroupId | M | T | F | T | F |

#### 5.3.58.3	Attribute constraints

None.

#### 5.3.58.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.59	UpfInfo <<dataType>>

#### 5.3.59.1	Definition

This data type represents a generic NFProfile definition (See TS 23.501 [22]).

#### 5.3.59.2	Attributes

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| smfServingAreas | O | T | T | F | T |

#### 5.3.59.3	Attribute constraints

None.

#### 5.3.59.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.60	AmfInfo <<dataType>>

#### 5.3.60.1	Definition

This data type represents AMF specific data in NFProfile definition (See TS 23.501 [22]).

#### 5.3.60.2	Attributes

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| priority | O | T | T | F | T |

#### 5.3.60.3	Attribute constraints

None.

#### 5.3.60.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.61	Udrinfo <<dataType>>

#### 5.3.61.1	Definition

This data type represents UDR specific data in NFProfile definition (See TS 23.501 [22]).

#### 5.3.61.2	Attributes

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| supportedDataSetIds | O | T | T | F | T |
| nFSrvGroupId | O | T | T | F | T |

#### 5.3.61.3	Attribute constraints

None.

#### 5.3.61.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.62	EP_N32

#### 5.3.62.1	Definition

This IOC represents an end point of N32 interface between cSEPP and pSEPP, which is defined in 3GPP TS 23.501 [2] and 33.501 [52].

#### 5.3.62.2	Attributes

The EP_N32 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| remotePlmnId | M | T | T | F | T |
| remoteSeppAddress | M | T | T | F | T |
| remoteSeppId | O | T | T | F | T |
| n32cParas | O | T | T | F | T |
| n32fPolicy | O | T | T | F | T |
| withIPX | M | T | T | F | T |

#### 5.3.62.3	Attribute constraints

None.

#### 5.3.62.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.63	ExternalSEPPFunction

#### 5.3.63.1	Definition

This IOC represents the properties, known by the management function, of a SEPP managed by another management function. For more information about SEPPFunction, see subclause 5.3.17.

#### 5.3.63.2	Attributes

The ExternalSEPPFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNId | M | T | F | F | T |
| sEPPId | M | T | F | T | T |
| fqdn | M | T | F | F | T |

#### 5.3.63.3	Attribute constraints

None.

#### 5.3.63.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.64	SEPPFunction <<ProxyClass>>

#### 5.3.64.1	Definition

This IOC represents an <<IOC>>SEPPFunction and <<IOC>>ExternalSEPPFunction.

#### 5.3.64.2	Attributes

See that defined in <<IOC>>SEPPFunction and <<IOC>>ExternalSEPPFunction.

#### 5.3.64.3	Attribute constraints

See respective IOCs.

#### 5.3.64.4	Notifications

See respective IOCs.

### 5.3.65	NEFFunction

#### 5.3.65.1	Definition

This IOC represents the NEF function in 5GC. For more information about the NEF, see 3GPP TS 23.501 [2].

#### 5.3.65.2	Attributes

The NEFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| sBIFQDN | M | T | T | F | T |
| sNSSAIList | CM | T | T | F | T |
| managedNFProfile | M | T | T | F | T |
| capabilityList | M | T | T | F | T |
| isINEF | O | T | F | T | F |
| isCAPIFSup | M | T | F | T | F |

#### 5.3.65.3	Attribute constraints

| Name | Definition |
| --- | --- |
| sNSSAIList Support Qualifier | Condition: Network slicing feature is supported. |

#### 5.3.65.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.66	SCPFunction

#### 5.3.67.1	Definition

This IOC represents a Service Communication Proxy, which is defined in 3GPP TS 23.501 [2].

#### 5.3.67.2	Attributes

The SCPFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| supportedFuncList | M | T | T | F | T |
| address | M | T | T | F | T |

#### 5.3.67.3	Attribute constraints

None.

#### 5.3.67.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.68	SupportedFunction <<dataType>>

#### 5.3.68.1	Definition

This dataType represents a functionality supported by a SCP, which is defined in 3GPP TS 23.501 [2].

#### 5.3.68.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| function | M | T | T | F | T |
| policy | O | T | T | F | T |

#### 5.3.68.3	Attribute constraints

None.

#### 5.3.68.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.69	CommModel <<dataType>>

#### 5.3.69.1	Definition

This data type represents a communication model definition (See TS 23.501 [22]).

#### 5.3.69.2	Attributes

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| groupId | M | T | T | F | T |
| commModelType | M | T | T | F | T |
| targetNFServiceList | M | T | T | F | T |
| commModelConfiguration | M | T | T | F | T |

#### 5.3.69.3	Attribute constraints

None

#### 5.3.69.4	Notifications

The subclause 5.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.70	QFQoSMonitoringControl

#### 5.3.70.1	Definition

This IOC specifies the capabilities and properties for control of QoS monitoring per QoS flow per UE for URLLC service. For more information about QoS monitoring per QoS flow per UE, see 3GPP TS 23.501 [2].

If the QoS monitoring per QoS flow per UE is enabled, the SMF requests the PSA UPF to perform the QoS monitoring per QoS flow per UE based on the attributes of the instance of this IOC.

#### 5.3.70.2	Attributes

The QFQoSMonitoringControl IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| qFQoSMonitoringState | M | T | T | F | T |
| qFMonitoredSNSSAIs | M | T | T | F | T |
| qFMonitored5QIs | M | T | T | F | T |
| isEventTriggeredQFMonitoringSupported | M | T | F | F | T |
| isPeriodicQFMonitoringSupported | M | T | F | F | T |
| isSessionReleasedQFMonitoringSupported | M | T | F | F | T |
| qFPacketDelayThresholds | CM | T | T | F | T |
| qFMinimumWaitTime | CM | T | T | F | T |
| qFMeasurementPeriod | CM | T | T | F | T |

#### 5.3.70.3	Attribute constraints

| Name | Definition |
| --- | --- |
| qFPacketDelayThresholds Support Qualifier | Condition: isEventTriggeredQFMonitoringSupported attribute of the same MOI is set to “Yes”. |
| qFMinimumWaitTime Support Qualifier | Condition: isEventTriggeredQFMonitoringSupported attribute of the same MOI is set to “Yes”. |
| qFMeasurementPeriod Support Qualifier | Condition: isPeriodicQFMonitoringSupported attribute of the same MOI is set to “Yes”. |

#### 5.3.70.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.71	QFDelayThresholdsType <<dataType>>

#### 5.3.71.1	Definition

This data type specifies the thresholds for reporting the packet delay for QoS monitoring per QoS flow per UE, see TS 29.244 [56].

#### 5.3.71.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| thresholdDl | M | T | T | F | T |
| thresholdUl | M | T | T | F | T |
| thresholdRtt | M | T | T | F | T |

#### 5.3.71.3	Attribute constraints

None

#### 5.3.71.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.72	GtpUPathQoSMonitoringControl

#### 5.3.72.1	Definition

This IOC specifies the capabilities and properties for control of GTP-U path QoS monitoring. For more information about the GTP-U path QoS monitoring, see 3GPP TS 23.501 [2].

If the GTP-U path QoS monitoring is enabled, the SMF requests the UPF(s) and NG-RAN to perform the GTP-U path QoS monitoring based on the attributes of the instance of this IOC.

#### 5.3.72.2	Attributes

The GtpUPathQoSMonitoringControl IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gtpUPathQoSMonitoringState | M | T | T | F | T |
| gtpUPathMonitoredSNSSAIs | M | T | T | F | T |
| monitoredDSCPs | M | T | T | F | T |
| isEventTriggeredGtpUPathMonitoringSupported | M | T | F | F | T |
| isPeriodicGtpUMonitoringSupported | M | T | F | F | T |
| isImmediateGtpUMonitoringSupported | M | T | F | F | T |
| gtpUPathDelayThresholds | CM | T | T | F | T |
| gtpUPathMinimumWaitTime | CM | T | T | F | T |
| gtpUPathMeasurementPeriod | CM | T | T | F | T |

#### 5.3.72.3	Attribute constraints

| Name | Definition |
| --- | --- |
| gtpUPathDelayThresholds Support Qualifier | Condition: isEventTriggeredGtpUPathMonitoringSupported attribute of the same MOI is set to “Yes”. |
| gtpUPathMinimumWaitTime Support Qualifier | Condition: isEventTriggeredGtpUPathMonitoringSupported attribute of the same MOI is set to “Yes”. |
| gtpUPathMeasurementPeriod Support Qualifier | Condition: isPeriodicGtpUMonitoringSupported attribute of the same MOI is set to “Yes”. |

#### 5.3.72.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.73	GtpUPathDelayThresholdsType <<dataType>>

#### 5.3.73.1	Definition

This data type specifies the thresholds for reporting the packet delay for GTP-U path QoS monitoring, see TS 29.244 [56].

#### 5.3.73.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| n3AveragePacketDelayThreshold | M | T | T | F | T |
| n3MinPacketDelayThreshold | M | T | T | F | T |
| n3MaxPacketDelayThreshold | M | T | T | F | T |
| n9AveragePacketDelayThreshold | M | T | T | F | T |
| n9MinPacketDelayThreshold | M | T | T | F | T |
| n9MaxPacketDelayThreshold | M | T | T | F | T |

#### 5.3.73.3	Attribute constraints

None

#### 5.3.73.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.75	Configurable5QISet

#### 5.3.75.1	Definition

This IOC specifies the pre-configured 5QIs, including their QoS characteristics,  see 3GPP TS 23.501 [2].

#### 5.3.75.2	Attributes

The Configurable5QISet IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| configurable5QIs | M | T | T | F | T |

#### 5.3.75.3	Attribute constraints

None.

#### 5.3.75.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.76	FiveQICharacteristics <<dataType>>

#### 5.3.76.1	Definition

This data type specifies the 5QI value and the corresponding QoS characteristics for a 5QI.

#### 5.3.76.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| fiveQIValue | M | T | T/F (NOTE) | F | T |
| resourceType | M | T | T/F (NOTE) | F | T |
| priorityLevel | O | T | T/F (NOTE) | F | T |
| packetDelayBudget | O | T | T/F (NOTE) | F | T |
| packetErrorRate | O | T | T/F (NOTE) | F | T |
| averagingWindow | O | T | T/F (NOTE) | F | T |
| maximumDataBurstVolume | O | T | T/F (NOTE) | F | T |
| NOTE: The isWritable qualifier is "T" if the attribute 1) describes a 5QI in Configurable5QISet MOI, or 2) describes a 5QI in Dynamic5QISet MOI which is associated to PCFFunction MOI or SMFFunction MOI when the PCF is not deployed; The isWritable qualifier is "F" otherwise. |  |  |  |  |  |

#### 5.3.76.3	Attribute constraints

None

#### 5.3.76.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.77	PacketErrorRate <<dataType>>

#### 5.3.77.1	Definition

This data type specifies the Packet Error Rate of a configurable 5QI.

#### 5.3.77.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| scalar | M | T | T/F (NOTE) | F | T |
| exponent | M | T | T/F (NOTE) | F | T |
| NOTE: The isWritable qualifier is “T” if the attribute 1) describes a 5QI in Configurable5QISet MOI, or 2) describes a 5QI in Dynamic5QISet MOI which is associated to PCFFunction MOI or SMFFunction MOI when the PCF is not deployed; The isWritable qualifier is "F" otherwise. |  |  |  |  |  |

#### 5.3.77.3	Attribute constraints

None

#### 5.3.77.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.78	FiveQiDscpMappingSet

#### 5.3.78.1	Definition

This IOC specifies the set of mapping between 5QIs and DSCP.

#### 5.3.78.2	Attributes

The FiveQiDscpMappingSet IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| fiveQiDscpMappingList | M | T | T | F | T |

#### 5.3.78.3	Attribute constraints

None.

#### 5.3.78.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.79	FiveQiDscpMapping <<dataType>>

#### 5.3.79.1	Definition

This data type specifies the mapping between 5QIs to DSCP.

#### 5.3.79.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| fiveQIValues | M | T | T | F | T |
| dscp | M | T | T | F | T |

#### 5.3.79.3	Attribute constraints

None

#### 5.3.79.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.80	PredefinedPccRuleSet

#### 5.3.80.1	Definition

This IOC specifies the predefined PCC rules, which are configured to SMF and referenced by PCF, see 3GPP TS 23.503 [59].

#### 5.3.80.2	Attributes

The PredefinedPccRuleSet IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| predefinedPccRules | M | T | T | F | T |

#### 5.3.80.3	Attribute constraints

None.

#### 5.3.80.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

### 5.3.81	PccRule <<dataType>>

#### 5.3.81.1	Definition

This data type specifies the PCC rule, see TS 29.512 [60].

#### 5.3.81.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pccRuleId | M | T | T | F | T |
| flowInfoList | CM | T | T | F | T |
| applicationId | CM | T | T | F | T |
| appDescriptor | O | T | T | F | T |
| contentVersion | O | T | T | F | T |
| precedence | CM | T | T | F | T |
| afSigProtocol | O | T | T | F | T |
| isAppRelocatable | O | T | T | F | T |
| isUeAddrPreserved | O | T | T | F | T |
| qosData | M | T | T | F | T |
| altQosParams | O | T | T | F | T |
| trafficControlData | M | T | T | F | T |
| conditionData | O | T | T | F | T |
| tscaiInputUl | O | T | T | F | T |
| tscaiInputDl | O | T | T | F | T |

#### 5.3.81.3	Attribute constraints

| Name | Definition |
| --- | --- |
| flowInfoList Support Qualifier | Condition: The applicationId is not supported. |
| applicationId Support Qualifier | Condition: The flowInfoList is not supported. |
| precedence Support Qualifier | Condition: The flowInfoList is provided. |

#### 5.3.81.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.82	FlowInformation <<dataType>>

#### 5.3.82.1	Definition

This data type specifies the flow information of a PCC rule.

#### 5.3.82.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| flowDescription | M | T | T | F | T |
| ethFlowDescription | M | T | T | F | T |
| packFiltId | M | T | T | F | T |
| packetFilterUsage | M | T | T | F | T |
| tosTrafficClass | M | T | T | F | T |
| spi | M | T | T | F | T |
| flowLabel | O | T | T | F | T |
| flowDirection | M | T | T | F | T |

#### 5.3.82.3	Attribute constraints

None

#### 5.3.82.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.83	EthFlowDescription <<dataType>>

#### 5.3.83.1	Definition

This data type describes an Ethernet flow.

#### 5.3.83.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| destMacAddr | M | T | T | F | T |
| ethType | M | T | T | F | T |
| fDesc | CM | T | T | F | T |
| fDir | M | T | T | F | T |
| sourceMacAddr | M | T | T | F | T |
| vlanTags | M | T | T | F | T |
| srcMacAddrEnd | O | T | T | F | T |
| destMacAddrEnd | O | T | T | F | T |

#### 5.3.83.3	Attribute constraints

| Name | Definition |
| --- | --- |
| fDesc Support Qualifier | Condition: The ethType is IP. |

#### 5.3.83.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.84	QoSData <<dataType>>

#### 5.3.84.1	Definition

This data type specifies the QoS control policy data for a service flow of a PCC rule.

#### 5.3.84.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| qosId | M | T | T | F | T |
| fiveQIValue | M | T | T | F | T |
| maxbrUl | O | T | T | F | T |
| maxbrDl | O | T | T | F | T |
| gbrUl | O | T | T | F | T |
| gbrDl | O | T | T | F | T |
| arp | M | T | T | F | T |
| qosNotificationControl | O | T | T | F | T |
| reflectiveQos | O | T | T | F | T |
| sharingKeyDl | O | T | T | F | T |
| sharingKeyUl | O | T | T | F | T |
| maxPacketLossRateDl | O | T | T | F | T |
| maxPacketLossRateUl | O | T | T | F | T |
| extMaxDataBurstVol | O | T | T | F | T |

#### 5.3.84.3	Attribute constraints

None.

#### 5.3.84.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.85	ARP <<dataType>>

#### 5.3.85.1	Definition

This data type specifies the allocation and retention priority of a QoS control policy.

#### 5.3.85.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| priorityLevel | M | T | T | F | T |
| preemptCap | M | T | T | F | T |
| preemptVuln | M | T | T | F | T |

#### 5.3.85.3	Attribute constraints

None

#### 5.3.85.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.86	TrafficControlData <<dataType>>

#### 5.3.86.1	Definition

This data type specifies the traffic control data for a service flow of a PCC rule.

#### 5.3.86.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| tcId | M | T | T | F | T |
| flowStatus | M | T | T | F | T |
| redirectInfo | O | T | T | F | T |
| addRedirectInfo | O | T | T | F | T |
| muteNotif | O | T | T | F | T |
| trafficSteeringPolIdDl | O | T | T | F | T |
| trafficSteeringPolIdUl | O | T | T | F | T |
| routeToLocs | M | T | T | F | T |
| traffCorreInd | O | T | T | F | T |
| upPathChgEvent | O | T | T | F | T |
| steerFun | O | T | T | F | T |
| steerModeDl | O | T | T | F | T |
| steerModeUl | O | T | T | F | T |
| mulAccCtrl | O | T | T | F | T |

#### 5.3.86.3	Attribute constraints

None

#### 5.3.86.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.87	RedirectInformation <<dataType>>

#### 5.3.87.1	Definition

This data type specifies the redirect information for traffic control in the PCC rule.

#### 5.3.87.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| redirectEnabled | M | T | T | F | T |
| redirectAddressType | M | T | T | F | T |
| redirectServerAddress | M | T | T | F | T |

#### 5.3.87.3	Attribute constraints

None

#### 5.3.87.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.88	RouteToLocation <<dataType>>

#### 5.3.88.1	Definition

This data type specifies a list of location which the traffic shall be routed to for the AF request.

#### 5.3.88.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dnai | M | T | T | F | T |
| routeInfo | CM | T | T | F | T |
| routeProfId | CM | T | T | F | T |

#### 5.3.88.3	Attribute constraints

| Name | Definition |
| --- | --- |
| routeInfo Support Qualifier | Condition: The routeProfId is not supported. |
| routeProfId Support Qualifier | Condition: The routeInfo is not supported. |

#### 5.3.88.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.89	RouteInformation <<dataType>>

#### 5.3.89.1	Definition

This data type specifies the traffic routing information.

#### 5.3.89.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| ipv4Addr | CM | T | T | F | T |
| ipv6Addr | CM | T | T | F | T |
| portNumber | M | T | T | F | T |

#### 5.3.89.3	Attribute constraints

| Name | Definition |
| --- | --- |
| ipv4Addr Support Qualifier | Condition: The ipv6Addr is not supported. |
| ipv6Addr Support Qualifier | Condition: The ipv4Addr is not supported. |

#### 5.3.89.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.90	UpPathChgEvent <<dataType>>

#### 5.3.90.1	Definition

This data type specifies the information about the AF subscriptions of the UP path change, see TS 29.512 [60].

#### 5.3.90.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| notificationUri | M | T | T | F | T |
| notifCorreId | M | T | T | F | T |
| dnaiChgType | M | T | T | F | T |
| afAckInd | O | T | T | F | T |

#### 5.3.90.3	Attribute constraints

None

#### 5.3.90.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.91	SteeringMode <<dataType>>

#### 5.3.91.1	Definition

This data type specifies the traffic distribution rule, see TS 29.512 [60].

#### 5.3.91.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| steerModeValue | M | T | T | F | T |
| active | CM | T | T | F | T |
| standby | O | T | T | F | T |
| threeGLoad | CM | T | T | F | T |
| prioAcc | CM | T | T | F | T |

#### 5.3.91.3	Attribute constraints

| Name | Definition |
| --- | --- |
| active Support Qualifier | Condition: The steerModeValue supports “ACTIVE_STANDBY”. |
| threeGLoad Support Qualifier | Condition: The steerModeValue supports “LOAD_BALANCING”. |
| prioAcc Support Qualifier | Condition: The steerModeValue supports “PRIORITY_BASED”. |

#### 5.3.91.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.92	ConditionData  <<dataType>>

#### 5.3.92.1	Definition

This data type specifies the condition data for a PCC rule.

#### 5.3.92.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| condId | M | T | T | F | T |
| activationTime | O | T | T | F | T |
| deactivationTime | O | T | T | F | T |
| accessType | O | T | T | F | T |
| ratType | O | T | T | F | T |

#### 5.3.92.3	Attribute constraints

None

#### 5.3.92.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.93	TscaiInputContainer  <<dataType>>

#### 5.3.93.1	Definition

This data type specifies the transports TSCAI input parameters for TSC traffic at the ingress interface of the DS-TT/UE for a PCC rule, see TS 29.512 [60].

#### 5.3.93.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| periodicity | O | T | T | F | T |
| burstArrivalTime | O | T | T | F | T |

#### 5.3.93.3	Attribute constraints

None

#### 5.3.93.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 5.3.94	Dynamic5QISet

#### 5.3.94.1	Definition

This IOC specifies the dynamically assigned 5QIs including their QoS characteristics, see 3GPP TS 23.501 [2]. The instance of this IOC shall not be created or modified by the MnS consumer except the instance is associated to PCFFunction MOI or SMFFunction MOI when the PCF is not deployed.

#### 5.3.94.2	Attributes

The Dynamic5QISet IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dynamic5QIs | M | T | T/F (NOTE) | F | T |
| NOTE: The isWritable qualifier is "T" if the Dynamic5QISet MOI is associated to PCFFunction MOI or SMFFunction MOI when the PCF is not deployed; The isWritable qualifier is "F" otherwise. |  |  |  |  |  |

#### 5.3.94.3	Attribute constraints

None.

#### 5.3.94.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

## 5.4	Attribute definitions

### 5.4.1	Attribute properties

The following table defines the attributes that are present in several Information Object Classes (IOCs) of the present document.

| Attribute Name |  |  |  | Documentation and Allowed Values |  |  |  | Properties |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| aMFIdentifier |  |  |  | The AMFI is constructed from an AMF Region ID, an AMF Set ID and an AMF Pointer. The AMF Region ID identifies the region, the AMF Set ID uniquely identifies the AMF Set within the AMF Region, and the AMF Pointer uniquely identifies the AMF within the AMF Set. (Ref. 3GPP TS 23.003 [13]) |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |  |  |
| aMFSetId |  |  |  | It represents the AMF Set ID, which is uniquely identifies the AMF Set within the AMF Region. allowedValues: defined in subclause 2.10.1 of 3GPP TS 23.003 [13]. |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |  |  |
| aMFSetMemberList |  |  |  | It is the list of DNs of AMFFunction instances of the AMFSet. allowedValues: N/A |  |  |  | type: DN multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |  |  |  |
| aMFRegionId |  |  |  | It represents the AMF Region ID, which identifies the region. allowedValues: defined in subclause 2.10.1 of 3GPP TS 23.003 [13]. |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |  |  |
| localAddress |  |  |  | This parameter specifies the localAddress including IP address and VLAN ID used for initialization of the underlying transport. First string is IP address, IP address can be an IPv4 address (See RFC 791 [37]) or an IPv6 address (See RFC 2373 [38]). Second string is VLAN Id (See IEEE 802.1Q [39]). |  |  |  | type: String multiplicity: 2 isOrdered: True isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| remoteAddress |  |  |  | Remote address including IP address used for initialization of the underlying transport. IP address can be an IPv4 address (See RFC 791 [37]) or an IPv6 address (See RFC 2373 [38]). |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| nfProfileList |  |  |  | It is a set of NFProfile(s) to be registered in the NRF instance. NFProfile is defined in 3GPP TS 29.510 [23]. |  |  |  | type: <<dataType>> multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |  |  |
| cNSIIdList |  |  |  | It is a set of NSI ID. NSI ID is an identifier for identifying the Core Network part of a Network Slice instance when multiple Network Slice instances of the same Network Slice are deployed, and there is a need to differentiate between them in the 5GC, see clause 3.1 of TS 23.501 [2] and subclause 6.1.6.2.7 of 3GPP TS 29.531 [24]. |  |  |  | type: String multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |  |  |
| sNSSAIList |  |  |  | See subclause 4.4.1. |  |  |  |  |  |  |  |
| sBIFQDN |  |  |  | It is used to indicate the FQDN of the registered NF instance in service-based interface, for example, NF instance FQDN structure is: nftype<nfnum>.slicetype<sliceid>.mnc<MNC>.mcc<MCC>.3gppnetwork.org |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |  |  |
| sBIServiceList |  |  |  | It is used to indicate the all supported NF services registered on service-based interface. |  |  |  | type: String multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |  |  |
| nRTACList |  |  |  | It is the list of Tracking Area Codes (either legacy TAC or extended TAC). allowedValues: Legacy TAC and Extended TAC are defined in clause 9.3.3.10 of TS 38.413 [5]. |  |  |  | type: Integer multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |  |  |
| supportedBMOList |  |  |  | It is used to indicate the list of supported BMOs (Bridge Managed Objects) required for integration with TSN system. |  |  |  | type: String multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |  |  |
| managedNFProfile |  |  |  | This parameter defines profile for managed NF (See TS 23.501 [22]). allowedValues: N/A |  |  |  | type: ManagedNFProfile multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |  |  |
| nfInstanceID |  |  |  | This parameter defines unique identity of the NF Instance. The format of the NF Instance ID shall be a Universally Unique Identifier (UUID) version 4, as described in IETF RFC 4122 [44] allowedValues: N/A |  |  |  | type: String multiplicity: 1 isOrdered: F isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| nfType |  |  |  | This parameter defines type of Network Function allowedValues: See TS 23.501[22] for NF types |  |  |  | type: ENUM multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| fqdn |  |  |  | This parameter defines FQDN of the Network Function (See TS 23.003 [5]) allowedValues: N/A |  |  |  | type: String multiplicity: 1 isOrdered: F isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| ipAddress |  |  |  | This parameter defines IP Address of the Network Function. It can be IPv4 address (See RFC 791 [37]) or IPv6 address (See RFC 2373 [38]). allowedValues: N/A |  |  |  | type: String multiplicity: 1 isOrdered: F isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| authzInfo |  |  |  | This parameter defines NF Specific Service authorization information. It shall include the NF type (s) and NF realms/origins allowed to consume NF Service(s) of NF Service Producer (See TS 23.501[22]). allowedValues: N/A |  |  |  | type: String multiplicity: 1 isOrdered: F isUnique: N/A defaultValue: None isNullable: True |  |  |  |
| locality |  |  |  | The parameter defines information about the location of the NF instance (e.g. geographic location, data center) defined by operator (See TS 29.510[23]). allowedValues: N/A |  |  |  | type: String multiplicity: 1 isOrdered: F isUnique: N/A defaultValue: None isNullable: True |  |  |  |
| capacity |  |  |  | This parameter defines static capacity information in the range of 0-65535, expressed as a weight relative to other NF instances of the same type; if capacity is also present in the nfServiceList parameters, those will have precedence over this value (See TS 29.510[23]) allowedValues: 0-65535 |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |  |  |
| nFInfo |  |  |  | This parameter includes NF specific data in Managed NF profile allowedValues: N/A |  |  |  | type: NFInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |  |  |
| hostAddr |  |  |  | This parameter defines host address of a NF allowedValues: N/A |  |  |  | type: HostAddr multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |  |  |
| priority |  |  |  | This parameter defines Priority (relative to other NFs of the same type) in the range of 0-65535, to be used for NF selection; lower values indicate a higher priority. If priority is also present in the nfServiceList parameters, those will have precedence over this value (See TS 29.510[23]). allowedValues: 0-65535 |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |  |  |
| supportedDataSetIds |  |  |  | This parameter defines list of supported data sets in the UDR instance (See TS 29.510[23]). allowedValues: "SUBSCRIPTION", "POLICY", EXPOSURE", "APPLICATION" |  |  |  | type: ENUM multiplicity: 1..* isOrdered: N/A isUnique: False defaultValue: None isNullable: False |  |  |  |
| nFSrvGroupId |  |  |  | This parameter defines identity of the group that is served by the NF instance (See TS 29.510[23]). allowedValues: N/A |  |  |  | type: String multiplicity: 1 isOrdered: F isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| smfServingAreas |  |  |  | This parameter defines the SMF service area(s) the UPF can serve (See TS 29.510[23]). allowedValues: N/A |  |  |  | type: String multiplicity: 1..* isOrdered: F isUnique: True defaultValue: None isNullable: False |  |  |  |
| isESCoveredBy |  |  |  | This indicates whether the adjacentCell provides no, partial or full coverage for the cell which name-contains the NRCellRelation instance. Adjacent cells with this attribute equal to "FULL" are recommended to be considered as candidate cells to take over the coverage when the original cell state is about to be changed to energySaving. All adjacent cells with this attribute value equal to "PARTIAL" are recommended to be considered as entirety of candidate cells to take over the coverage when the original cell state is about to be changed to energySaving. allowedValues: NO, PARTIAL, FULL |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| commModelList |  |  |  | The attribute specifies a list of commModel which is defined as a datatype (see clause 5.3.69). It can be used by NF and NF services to interact with each other in 5G Core network (see TS 23.501 [2]). allowedValues: Not applicable |  |  |  | type: commModel multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| groupId |  |  |  | This parameter identiies a list of target NF services on which the same communication model is applied to. allowedValues: N/A |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: False defaultValue: None isNullable: False |  |  |  |
| commModelType |  |  |  | This parameter defines communication model used by a NF to interact with NF service(s) (See TS 23.501 [2]). allowedValues:”DIRECT_COMMUNICATION_WO_NRF”, “DIRECT_COMMUNICATION_WITH_NRF”, “INDIRECT_COMMUNICATION_WO_DEDICATED_DISCOVERY”, “INDIRECT_COMMUNICATION_WITH_DEDICATED_DISCOVERY” |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |  |  |
| targetNFServiceList |  |  |  | This parameter lists target NF services sharing same communication model and configuration. allowedValues: N/A |  |  |  | type: DN multiplicity: 1..* isOrdered: F isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| commModelConfiguration |  |  |  | This parameter defines configuration parameters for specific communication model for a group of NF Services. allowedValues: N/A |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |  |  |
| supportedFuncList |  |  |  | This parameter lists functionalities supported by a SCP. Refer to TS 23.501 [2]. |  |  |  | type: SupportedFunction multiplicity: 1..* isOrdered: N/A isUnique: False defaultValue: None isNullable: False |  |  |  |
| address |  |  |  | This parameter defines address of a SCP instance, it can be IP address (either IPv4 address (See RFC 791 [37]) or IPv6 address (See RFC 2373 [38])) or FQDN (See TS 23.003 [5]). |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |  |  |
| function |  |  |  | This parameter defines name of a functionality supported by a SCP. |  |  |  | type: String multiplicity: 1 isOrdered: F isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| policy |  |  |  | This parameter defines configuration policies of a functionality supported by a SCP. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |  |  |
| capabilityList |  |  |  | This parameter lists capabilities supported by a NEF. Refer to TS 23.501 [2]. allowedValues: N/A |  |  |  | type: String multiplicity: 1..* isOrdered: N/A isUnique: False defaultValue: None isNullable: False |  |  |  |
| isINEF |  |  |  | This parameter defines if the NEF is an Intermediate NEF. allowedValues: TRUE, FALSE |  |  |  | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |  |  |
| isCAPIFSup |  |  |  | This parameter defines if the NEF support Common API Framework. allowedValues: TRUE, FALSE |  |  |  | type: Boolean multiplicity: 1 isOrdered: F isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| sEPPType |  |  |  | This parameter defines the type of a SEPP entity. Refer to TS 33.501 [52]. allowedValues: “CSEPP”, “PSEPP” |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: False defaultValue: None isNullable: False |  |  |  |
| sEPPId |  |  |  | This parameter is identifier of a SEPP, it is unique inside a PLMN. allowedValues: N/A |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |  |  |
| remotePlmnId |  |  |  | This parameter defines PLMNId of the remote SEPP. allowedValues: N/A |  |  |  | Type: PLMNId multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| remoteSeppAddress |  |  |  | This parameter defines address of the remote SEPP. It can be IP address (either IPv4 address (See RFC 791 [37]) or IPv6 address (See RFC 2373 [38])) or FQDN(See TS 23.003 [5]). allowedValues: N/A |  |  |  | type: String multiplicity: 1 isOrdered: F isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| remoteSeppId |  |  |  | This parameter defines identifier of the remote SEPP. it is unique inside a PLMN. allowedValues: N/A |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |  |  |
| n32cParas |  |  |  | This attribute is used to configure parameters to establish security link between two SEPPs. allowedValues: N/A |  |  |  | type: String multiplicity: 1 isOrdered: F isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| n32fPolicy |  |  |  | This attribute is used to configure policies to protect the messages exchanged between SEPPs. allowedValues: N/A |  |  |  | type: String multiplicity: 1 isOrdered: F isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| withIPX |  |  |  | This attribute defines if there’s an IPX interconnected between two SEPPs. allowedValues: TRUE, FALSE |  |  |  | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |  |  |  |
| FiveQiDscpMappingList |  |  |  | It provides the list of mapping between 5QIs and DSCP. allowedValues: N/A |  |  |  | type: FiveQiDscpMapping multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| fiveQIValues |  |  |  | It indicates a list of 5QI value. allowedValues: 0 - 255 |  |  |  | type: Integer multiplicity: * isOrdered: N/A isUnique: Yes defaultValue: None isNullable: False |  |  |  |
| dscp |  |  |  | It indicates a DSCP. allowedValues: 0 - 255 |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: Yes defaultValue: None isNullable: False |  |  |  |
| configurable5QISetRef |  |  |  | This is the DN of Configurable5QISet. allowedValues: DN of the Configurable5QISet MOI. |  |  |  | type: String multiplicity: 0..1 isOrdered: False isUnique: True defaultValue: None isNullable: True |  |  |  |
| configurable5QIs |  |  |  | It indicates the pre-configured 5QIs, including their QoS characteristics. allowedValues: N/A |  |  |  | type: FiveQICharacteristics multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| dynamic5QISetRef |  |  |  | This is the DN of Dynamic5QISet MOI. allowedValues: DN of the Dynamic5QISet MOI. |  |  |  | type: String multiplicity: 0..1 isOrdered: False isUnique: True defaultValue: None isNullable: True |  |  |  |
| dynamic5QIs |  |  |  | It indicates the dynamically assigned 5QIs, including their QoS characteristics. allowedValues: N/A |  |  |  | type: FiveQICharacteristics multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| fiveQIValue |  |  |  | It identifies the 5QI value. allowedValues: 0 - 255 |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: Yes defaultValue: None isNullable: False |  |  |  |
| resourceType |  |  |  | It indicates the Resource Type of a 5QI, as specified in TS 23.501 [2]. allowedValues: “GBR”, “Non-GBR” |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: False defaultValue: None isNullable: False |  |  |  |
| priorityLevel |  |  |  | It indicates the Priority Level of a 5QI, as specified in TS 23.501 [2]. allowedValues: 0 - 127 |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: False defaultValue: None isNullable: False |  |  |  |
| packetDelayBudget |  |  |  | It indicates the Packet Delay Budget (in unit of 0.5ms) of a 5QI, as specified in TS 23.501 [2]. allowedValues: 0 - 1023 |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: False defaultValue: None isNullable: False |  |  |  |
| packetErrorRate |  |  |  | It indicates the Packet Error Rate of a 5QI, as specified in TS 23.501 [2]. allowedValues: N/A |  |  |  | type: PacketErrorRate multiplicity: 1 isOrdered: N/A isUnique: False defaultValue: None isNullable: False |  |  |  |
| averagingWindow |  |  |  | It indicates the Averaging Window (in unit of ms) of a 5QI, as specified in TS 23.501 [2]. allowedValues: 0 - 4095 |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: False defaultValue: None isNullable: False |  |  |  |
| maximumDataBurstVolume |  |  |  | It indicates the Maximum Data Burst Volume (in unit of Byte) of a 5QI, as specified in TS 23.501 [2]. allowedValues: 0 - 4095 |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: False defaultValue: None isNullable: False |  |  |  |
| scalar |  |  |  | The Packet Error Rate of a 5QI expressed as Scalar x 10-k where k is the Exponent. This attriutes indicates the Scalar of this expression. allowedValues: 0 - 9 |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: False defaultValue: None isNullable: False |  |  |  |
| exponent |  |  |  | The Packet Error Rate of a 5QI expressed as Scalar x 10-k where k is the Exponent. This attriutes indicates the Exponent of this expression. allowedValues: 0 - 9 |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: False defaultValue: None isNullable: False |  |  |  |
| gtpUPathQoSMonitoringState |  |  |  | It indicates the state of GTP-U path QoS monitoring for URLLC service. allowedValues: "Enabled", "Disabled". |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Enabled isNullable: False |  |  |  |
| gtpUPathMonitoredSNSSAIs |  |  |  | It specifies the S-NSSAIs for which the GTP-U path QoS monitoring is to be performed. allowedValues: See 3GPP TS 23.003 [13] |  |  |  | type: S-NSSAI multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| monitoredDSCPs |  |  |  | It specifies the DSCPs for which the GTP-U path QoS monitoring is to be performed. allowedValues: See 3GPP TS 29.244 [56] |  |  |  | type: Integer multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| isEventTriggeredGtpUPathMonitoringSupported |  |  |  | It indicates whether the event triggered GTP-U path QoS monitoring reporting based on thresholds is supported, see 3GPP TS 29.244 [56]. allowedValues: “Yes”, “No”. |  |  |  | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Yes isNullable: False |  |  |  |
| isPeriodicGtpUMonitoringSupported |  |  |  | It indicates whether the periodic GTP-U path QoS monitoring reporting is supported, see 3GPP TS 29.244 [56]. allowedValues: “Yes”, “No”. |  |  |  | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Yes isNullable: False |  |  |  |
| isImmediateGtpUMonitoringSupported |  |  |  | It indicates whether the immediate GTP-U path QoS monitoring reporting is supported, see 3GPP TS 29.244 [56]. allowedValues: “Yes”, “No”. |  |  |  | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Yes isNullable: False |  |  |  |
| gtpUPathDelayThresholds |  |  |  | It specifies the thresholds for reporting the packet delay for the GTO-U path QoS monitoring, if the isEventTriggeredGtpUPathMonitoringSupported attribute of the same MOI is set to “yes”. The packet delay will be reported to SMF when it exceeds the threshold (in milliseconds). allowedValues: N/A. |  |  |  | type: GtpUPathDelayThresholdsType multiplicity: 1 isOrdered: Y isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| gtpUPathMinimumWaitTime |  |  |  | It specifies the minimum waiting time (in seconds) between two consecutive reports for event triggered GTP-U path QoS monitoring reporting, if the isEventTriggeredGtpUPathMonitoringSupported attribute of the same MOI is set to “yes”. allowedValues: see 3GPP TS 29.244 [56]. |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| gtpUPathMeasurementPeriod |  |  |  | It specifies the period (in seconds) for reporting the packet delay for GTP-U path QoS monitoring, if the isPeriodicGtpUMonitoringSupported attribute of the same MOI is set to “yes”. allowedValues: see 3GPP TS 29.244 [56]. |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| n3AveragePacketDelayThreshold |  |  |  | It specifies the threshold for reporting the average packet delay of a GTP-U path on N3 interface. allowedValues: see 3GPP TS 29.244 [56]. |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| n3MinPacketDelayThreshold |  |  |  | It specifies the threshold for reporting the minimum packet delay of a GTP-U path on N3 interface. allowedValues: see 3GPP TS 29.244 [56]. |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| n3MaxPacketDelayThreshold |  |  |  | It specifies the threshold for reporting the maxinum packet delay of a GTP-U path on N3 interface. allowedValues: see 3GPP TS 29.244 [56]. |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| n9AveragePacketDelayThreshold |  |  |  | It specifies the threshold for reporting the average packet delay of a GTP-U path on N9 interface. allowedValues: see 3GPP TS 29.244 [56]. |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| n9MinPacketDelayThreshold |  |  |  | It specifies the threshold for reporting the minimum packet delay of a GTP-U path on N9 interface. allowedValues: see 3GPP TS 29.244 [56]. |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| n9MaxPacketDelayThreshold |  |  |  | It specifies the threshold for reporting the maxinum packet delay of a GTP-U path on N9 interface. allowedValues: see 3GPP TS 29.244 [56]. |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| qFQoSMonitoringState |  |  |  | It indicates the state of QoS monitoring per QoS flow per UE for URLLC service. allowedValues: "Enabled", "Disabled". |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Enabled isNullable: False |  |  |  |
| qFMonitoredSNSSAIs |  |  |  | It specifies the S-NSSAIs for which the QoS monitoring per QoS flow per UE is to be performed. allowedValues: See 3GPP TS 23.003 [13] |  |  |  | type: S-NSSAI multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| qFMonitored5QIs |  |  |  | It specifies the 5QIs for which the QoS monitoring per QoS flow per UE is to be performed. allowedValues: See 3GPP TS 23.501[2] |  |  |  | type: Integer multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| isEventTriggeredQFMonitoringSupported |  |  |  | It indicates whether the event based QoS monitoring reporting per QoS flow per UE is supported, see 3GPP TS 29.244 [56]. allowedValues: “Yes”, “No”. |  |  |  | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Yes isNullable: False |  |  |  |
| isPeriodicQFMonitoringSupported |  |  |  | It indicates whether the periodic QoS monitoring reporting per QoS flow per UE is supported, see 3GPP TS 29.244 [56]. allowedValues: “Yes”, “No”. |  |  |  | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Yes isNullable: False |  |  |  |
| isSessionReleasedQFMonitoringSupported |  |  |  | It indicates whether the session release based QoS monitoring reporting per QoS flow per UE is supported, see 3GPP TS 29.244 [56]. allowedValues: “Yes”, “No”. |  |  |  | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Yes isNullable: False |  |  |  |
| qFPacketDelayThresholds |  |  |  | It specifies the thresholds for reporting the packet delay between PSA and UE for QoS monitoring per QoS flow per UE, if the isEventTriggeredQFMonitoringSupported attribute of the same MOI is set to “yes”.”. The packet delay will be reported by PSA UPF to SMF when it exceeds the threshold (in milliseconds). allowedValues: see 3GPP TS 29.244 [56]. |  |  |  | type: QFPacketDelayThresholdsType multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| qFMinimumWaitTime |  |  |  | It specifies the minimum waiting time (in seconds) between two consecutive reports for event triggered QoS monitoring reporting per QoS flow per UE, if the isEventTriggeredQFMonitoringSupported attribute of the same MOI is set to “yes”. allowedValues: see 3GPP TS 29.244 [56]. |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| qFMeasurementPeriod |  |  |  | It specifies the period (in seconds) for reporting the packet delay for QoS monitoring per QoS flow per UE, if the isPeriodicQFMonitoringSupported attribute of the same MOI is set to “yes”. allowedValues: see 3GPP TS 29.244 [56]. |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| thresholdDl |  |  |  | It specifies the threshold for reporting the DL packet delay between PSA UPF and UE. allowedValues: see 3GPP TS 29.244 [56]. |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| thresholdUl |  |  |  | It specifies the threshold for reporting the UL packet delay between PSA UPF and UE. allowedValues: see 3GPP TS 29.244 [56]. |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| thresholdRtt |  |  |  | It specifies the threshold for reporting the round-trip packet delay between PSA UPF and UE. allowedValues: see 3GPP TS 29.244 [56]. |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| predefinedPccRules |  |  |  | It specifies the predefined PCC Rules, see TS 25.503 [59]. allowedValues: N/A |  |  |  | type: PccRule multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| pccRuleId |  |  |  | It identifies the PCC rule. allowedValues: N/A |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| flowInfoList |  |  |  | It is a list of IP flow packet filter information. allowedValues: N/A |  |  |  | type: FlowInformation multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| applicationId |  |  |  | A reference to the application detection filter configured at the UPF. allowedValues: N/A |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| appDescriptor |  |  |  | It is the ATSSS rule application descriptor. allowedValues: see TS 29.571 [61]. |  |  |  | type: BitString multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| contentVersion |  |  |  | Indicates the content version of the PCC rule. allowedValues: N/A |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| precedence |  |  |  | It indicates the order in which this PCC rule is applied relative to other PCC rules within the same PDU session. allowedValues: 0..255. |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| afSigProtocol |  |  |  | Indicates the protocol used for signalling between the UE and the AF. The default value is "NO_INFORMATION". allowedValues: “NO_INFORMATION”, “SIP”. |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: “NO_INFORMATION” isNullable: False |  |  |  |
| isAppRelocatable |  |  |  | It indicates the application relocation possibility. The default value is "FALSE. allowedValues: “TRUE”, “FALSE”. |  |  |  | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| isUeAddrPreserved |  |  |  | It Indicates whether UE IP address should be preserved. The default value is "FALSE". allowedValues: “TRUE”, “FALSE”. |  |  |  | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: “FALSE” isNullable: False |  |  |  |
| qosData |  |  |  | It contains the QoS control policy data for a PCC rule. allowedValues: N/A |  |  |  | type: QoSData multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| altQosParams |  |  |  | It contains the QoS control policy data for the Alternative QoS parameter sets of the service data flow. Only the "qosId" attribute, "5qi" attribute, "maxbrUl" attribute, "maxbrDl" attribute, "gbrUl" attribute and "gbrDl" attribute are applicable within the QosData data type. This data type represents an ordered list, where the lower the index of the array for a given entry, the higher the priority. allowedValues: N/A |  |  |  | type: QoSData multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| trafficControlData |  |  |  | It contains the traffic control policy data for a PCC rule. allowedValues: N/A |  |  |  | type: TrafficControlData multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| conditionData |  |  |  | It contains the condition data for a PCC rule. allowedValues: N/A |  |  |  | type: ConditionData multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| tscaiInputUl |  |  |  | It contains transports TSCAI input parameters for TSC traffic at the ingress interface of the DS-TT/UE (uplink flow direction). allowedValues: N/A |  |  |  | type: TscaiInputContainer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| tscaiInputDl |  |  |  | It contains transports TSCAI input parameters for TSC traffic at the ingress of the NW-TT (downlink flow direction). allowedValues: N/A |  |  |  | type: TscaiInputContainer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| flowDescription |  |  |  | It defines a packet filter for an IP flow. allowedValues: see TS 29.214 [62]. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| ethFlowDescription |  |  |  | It defines a packet filter for an Ethernet flow. allowedValues: see TS 29.514 [62]. |  |  |  | type: EthFlowDescription multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| destMacAddr |  |  |  | It specifies the destination MAC address formatted in the hexadecimal notation according to clause 1.1 and clause 2.1 of IETF RFC 7042 [63]. Pattern: '^([0-9a-fA-F]{2})((-[0-9a-fA-F]{2}){5})$'. allowedValues: N/A. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| ethType |  |  |  | A two-octet string that represents the Ethertype, as described in IEEE 802.3 [64] and IETF RFC 7042 [63] in hexadecimal representation. Each character in the string shall take a value of "0" to "9" or "A" to "F" and shall represent 4 bits. The most significant character representing the 4 most significant bits of the ethType shall appear first in the string, and the character representing the 4 least significant bits of the ethType shall appear last in the string. allowedValues: see IEEE 802.3 [64] and IETF RFC 7042 [63]. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| fDesc |  |  |  | It contains the flow description for the Uplink or Downlink IP flow. It shall be present when the ethtype is IP. allowedValues: see flowDescription in TS 29.214 [62]. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| fDir |  |  |  | It indicates the packet filter direction. allowedValues: "DOWNLINK", "UPLINK". |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| sourceMacAddr |  |  |  | It specifies the source MAC address formatted in the hexadecimal notation according to clause 1.1 and clause 2.1 of IETF RFC 7042 [63]. Pattern: '^([0-9a-fA-F]{2})((-[0-9a-fA-F]{2}){5})$'. allowedValues: N/A. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| vlanTags |  |  |  | It specifies the Customer-VLAN and/or Service-VLAN tags containing the VID, PCP/DEI fields as defined in IEEE 802.1Q [65] and IETF RFC 7042 [63]. The first/lower instance in the array stands for the Customer-VLAN tag and the second/higher instance in the array stands for the Service-VLAN tag. Each field is encoded as a two-octet string in hexadecimal representation. Each character in the string shall take a value of "0" to "9" or "A" to "F" and shall represent 4 bits. The most significant character representing the PCP/DEI field shall appear first in the string, followed by character representing the 4 most significant bits of the VID field, and the character representing the 4 least significant bits of the VID field shall appear last in the string. If only Service-VLAN tag is provided, empty string for Customer-VLAN tag shall be provided. allowedValues: see IEEE 802.1Q [65] and IETF RFC 7042 [63]. |  |  |  | type: String multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| srcMacAddrEnd |  |  |  | It specifies the source MAC address end. If this attribute is present, the sourceMacAddr attribute specifies the source MAC address start. E.g. srcMacAddrEnd with value 00-10-A4-23-3E-FE and sourceMacAddr with value 00-10-A4-23-3E-02 means all MAC addresses from 00-10-A4-23-3E-02 up to and including 00-10-A4-23-3E-FE. allowedValues: N/A. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |  |  |  |
| destMacAddrEnd |  |  |  | It specifies the destination MAC address end. If this attribute is present, the destMacAddr attribute specifies the destination MAC address start. allowedValues: N/A. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |  |  |  |
| packFiltId |  |  |  | It is the identifier of the packet filter. allowedValues: N/A. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| packetFilterUsage |  |  |  | It indicates if the packet shall be sent to the UE. The default value is "FALSE". allowedValues: TRUE, FALSE |  |  |  | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: “FALSE” isNullable: False |  |  |  |
| tosTrafficClass |  |  |  | It contains the Ipv4 Type-of-Service and mask field or the Ipv6 Traffic-Class field and mask field. allowedValues: N/A |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| spi |  |  |  | It is the security parameter index of the IPSec packet, see IETF RFC 4301 [66]. allowedValues: see IETF RFC 4301 [66]. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |  |  |  |
| flowLabel |  |  |  | It specifies the Ipv6 flow label header field. AllowedValues: N/A |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |  |  |  |
| flowDirection |  |  |  | It indicates the direction/directions that a filter is applicable. AllowedValues: “DOWNLINK”, “UPLINK”, “BIDIRECTIONAL”, “UNSPECIFIED”. |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |  |  |  |
| qosId |  |  |  | It identifies the QoS control policy data for a PCC rule. AllowedValues: N/A |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| maxbrUl |  |  |  | It represents the maximum uplink bandwidth formatted as follows: Pattern: '^\d+(\.\d+)? (bps¦Kbps¦Mbps¦Gbps¦Tbps)$', see TS 29.512 [60]. Examples: "125 Mbps", "0.125 Gbps", "125000 Kbps" AllowedValues: N/A |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |  |  |  |
| maxbrDl |  |  |  | It represents the maximum downlink bandwidth formatted as follows: Pattern: '^\d+(\.\d+)? (bps¦Kbps¦Mbps¦Gbps¦Tbps)$', see TS 29.512 [60]. Examples: "125 Mbps", "0.125 Gbps", "125000 Kbps". AllowedValues: N/A. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |  |  |  |
| gbrUl |  |  |  | It represents the guaranteed uplink bandwidth formatted as follows: Pattern: '^\d+(\.\d+)? (bps¦Kbps¦Mbps¦Gbps¦Tbps)$', see TS 29.512 [60]. Examples: "125 Mbps", "0.125 Gbps", "125000 Kbps". AllowedValues: N/A. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |  |  |  |
| gbrDl |  |  |  | It represents the guaranteed downlink bandwidth formatted as follows: Pattern: '^\d+(\.\d+)? (bps¦Kbps¦Mbps¦Gbps¦Tbps)$', see TS 29.512 [60]. Examples: "125 Mbps", "0.125 Gbps", "125000 Kbps". AllowedValues: N/A. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |  |  |  |
| extMaxDataBurstVol |  |  |  | It denotes the largest amount of data that is required to be transferred within a period of 5G-AN PDB, see TS 29.512 [60]. AllowedValues: 4096..2000000. |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |  |  |  |
| arp |  |  |  | It indicates the allocation and retention priority. AllowedValues: N/A. |  |  |  | type: ARP multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| ARP.priorityLevel |  |  |  | It defines the relative importance of a resource request. AllowedValues: 1..15. |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| preemptCap |  |  |  | It defines whether a service data flow may get resources that were already assigned to another service data flow with a lower priority level. AllowedValues: "NOT_PREEMPT", "MAY_PREEMPT". |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| preemptVuln |  |  |  | It defines whether a service data flow may lose the resources assigned to it in order to admit a service data flow with higher priority level. AllowedValues: "NOT_PREEMPTABLE", "PREEMPTABLE". |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| qosNotificationControl |  |  |  | It indicates whether notifications are requested from 3GPP NG-RAN when the GFBR can no longer (or again) be guaranteed for a QoS Flow during the lifetime of the QoS Flow. The default value is "FALSE". AllowedValues: "TRUE", "FALSE". |  |  |  | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: “FALSE” isNullable: False |  |  |  |
| reflectiveQos |  |  |  | Indicates whether the QoS information is reflective for the corresponding non-GBR service data flow. The default value is "FALSE". AllowedValues: "TRUE", "FALSE". |  |  |  | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: “FALSE” isNullable: False |  |  |  |
| sharingKeyDl |  |  |  | It indicates, by containing the same value, what PCC rules may share resource in downlink direction. AllowedValues: N/A. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |  |  |  |
| sharingKeyUl |  |  |  | It indicates, by containing the same value, what PCC rules may share resource in uplink direction. AllowedValues: N/A. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |  |  |  |
| maxPacketLossRateDl |  |  |  | It indicates the downlink maximum rate for lost packets that can be tolerated for the service data flow. AllowedValues: 0..1000. |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |  |  |  |
| maxPacketLossRateUl |  |  |  | It indicates the uplink maximum rate for lost packets that can be tolerated for the service data flow. AllowedValues: 0..1000. |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |  |  |  |
| tcId |  |  |  | It univocally identifies the traffic control policy data within a PDU session. AllowedValues: N/A. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| flowStatus |  |  |  | It represents whether the service data flow(s) are enabled or disabled. The default value is "ENABLED". See TS 29.514 [67]. AllowedValues: “ENABLED-UPLINK”, “ENABLED-DOWNLINK”, “ENABLED”, “DISABLED”, “REMOVED”. |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: “ENABLED” isNullable: False |  |  |  |
| redirectInfo |  |  |  | It indicates whether the detected application traffic should be redirected to another controlled address. AllowedValues: N/A. |  |  |  | type: RedirectInformation multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: “ENABLED” isNullable: False |  |  |  |
| addRedirectInfo |  |  |  | It contains the additional redirect information indicating whether the detected application traffic should be redirected to another controlled address. AllowedValues: N/A. |  |  |  | type: RedirectInformation multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: “ENABLED” isNullable: False |  |  |  |
| redirectEnabled |  |  |  | It indicates whether the redirect instruction is enabled. AllowedValues: "TRUE", "FALSE". |  |  |  | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| redirectAddressType |  |  |  | It indicates the type of redirect address, see TS 29.512 [60]. AllowedValues: " IPV4_ADDR", "IPV6_ADDR", “URL”, “SIP_URI”. |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| redirectServerAddress |  |  |  | It indicates the address of the redirect server. AllowedValues: N/A. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| muteNotif |  |  |  | It indicates whether applicat'on's start or stop notification is to be muted. The default value is "FALSE". AllowedValues: "TRUE", "FALSE". |  |  |  | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: “FALSE” isNullable: False |  |  |  |
| trafficSteeringPolIdDl |  |  |  | It references to a pre-configured traffic steering policy for downlink traffic at the SMF, see TS 29.512 [60]. AllowedValues: N/A. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| trafficSteeringPolIdUl |  |  |  | It references to a pre-configured traffic steering policy for uplink traffic at the SMF, see TS 29.512 [60]. AllowedValues: N/A. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| routeToLocs |  |  |  | It provides a list of location which the traffic shall be routed to for the AF request. AllowedValues: N/A. |  |  |  | type: RouteToLocation multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| traffCorreInd |  |  |  | It indicates the traffic correlation. AllowedValues: "TRUE", "FALSE". |  |  |  | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: “FALSE” isNullable: False |  |  |  |
| dnai |  |  |  | It represents the DNAI (Data network access identifier), see 3GPP TS 23.501 [2]. AllowedValues: N/A. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| routeInfo |  |  |  | It provides the traffic routing information. AllowedValues: N/A. |  |  |  | type: RouteInformation multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| ipv4Addr |  |  |  | It defines the Ipv4 address of the tunnel end point in the data network, formatted in the "dotted decimal" notation. Pattern: '^(([0-9]¦[1-9][0-9]¦1[0-9][0-9]¦2[0-4][0-9]¦25[0-5])\.){3}([0-9]¦[1-9][0-9]¦1[0-9][0-9]¦2[0-4][0-9]¦25[0-5])$'. AllowedValues: N/A. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| ipv6Addr |  |  |  | It defines the Ipv6 address of the tunnel end point in the data network. Pattern: '^((:¦(0?¦([1-9a-f][0-9a-f]{0,3}))):)((0?¦([1-9a-f][0-9a-f]{0,3})):){0,6}(:¦(0?¦([1-9a-f][0-9a-f]{0,3})))$' and Pattern: '^((([^:]+:){7}([^:]+))¦((([^:]+:)*[^:]+)?::(([^:]+:)*[^:]+)?))$'. AllowedValues: N/A. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| portNumber |  |  |  | It defines the UDP port number of the tunnel end point in the data network, see TS 29.571 [61]. AllowedValues: N/A. |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| routeProfId |  |  |  | It identifies the routing profile. AllowedValues: N/A. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| upPathChgEvent |  |  |  | It contains the information about the AF subscriptions of the UP path change. AllowedValues: N/A. |  |  |  | type: UpPathChgEvent multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| notificationUri |  |  |  | It provides notification address (Uri) of AF receiving the event notification. AllowedValues: N/A. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| notifCorreId |  |  |  | It is used to set the value of Notification Correlation ID in the notification sent by the SMF, see TS 29.512 [60]. AllowedValues: N/A. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| dnaiChgType |  |  |  | It indicates the type of DNAI change, see TS 29.512 [60]. AllowedValues: “EARLY”, “EARLY_LATE”, “LATE”. |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| afAckInd |  |  |  | It identifies whether the AF acknowledgement of UP path event notification is expected.The default value is "FALSE". AllowedValues: “TRUE”, “FALSE”. |  |  |  | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: “FALSE” isNullable: False |  |  |  |
| steerFun |  |  |  | It indicates the applicable traffic steering functionality, see TS 29.512 [60]. AllowedValues: “MPTCP”, “ATSSS_LL”. |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| steerModeDl |  |  |  | It provides the traffic distribution rule across 3GPP and Non-3GPP accesses to apply for downlink traffic. AllowedValues: N/A. |  |  |  | type: SteeringMode multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| steerModeUl |  |  |  | It provides the traffic distribution rule across 3GPP and Non-3GPP accesses to apply for uplink traffic. AllowedValues: N/A. |  |  |  | type: SteeringMode multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| mulAccCtrl |  |  |  | It indicates whether the service data flow, corresponding to the service data flow template, is allowed or not allowed. The default value is "NOT_ALLOWED". AllowedValues: "ALLOWED", "NOT_ALLOWED". |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "NOT_ALLOWED" isNullable: False |  |  |  |
| steerModeValue |  |  |  | It indicates the value of the steering mode, see TS 29.512 [60]. AllowedValues: “ACTIVE_STANDBY”, “LOAD_BALANCING”, “SMALLEST_DELAY”, “PRIORITY_BASED”. |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| active |  |  |  | It indicates the active access, see TS 29.571 [61]. AllowedValues: "3GPP_ACCESS", "NON_3GPP_ACCESS". |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| standby |  |  |  | It indicates the Standby access, see TS 29.571 [61]. AllowedValues: "3GPP_ACCESS", "NON_3GPP_ACCESS". |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| threeGLoad |  |  |  | It indicates the traffic load to steer to the 3GPP Access expressed in one percent. AllowedValues: 0..100. |  |  |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| prioAcc |  |  |  | It indicates the high priority access, see TS 29.571 [61]. AllowedValues: "3GPP_ACCESS", "NON_3GPP_ACCESS". |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| condId |  |  |  | It uniquely identifies the condition data. AllowedValues: N/A. |  |  |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| activationTime |  |  |  | It indicates the time (in date-time format) when the decision data shall be activated, see TS 29.512 [60] and TS 29.571 [61]. AllowedValues: N/A. |  |  |  | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| deactivationTime |  |  |  | It indicates the time (in date-time format) when the decision data shall be deactivated, see TS 29.512 [60] and TS 29.571 [61]. AllowedValues: N/A. |  |  |  | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| accessType |  |  |  | It provides the condition of access type of the UE when the session AMBR shall be enforced, see TS 29.512 [60]. AllowedValues: "3GPP_ACCESS", "NON_3GPP_ACCESS". |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| ratType |  |  |  | It provides the condition of RAT type of the UE when the session AMBR shall be enforced, see TS 29.512 [60] and TS 29.571 [61]. AllowedValues: "NR", "EUTRA", “WLAN”, “VIRTUAL”, “NBIOT”, “WIRELINE”, “WIRELINE_CABLE”, “WIRELINE_BBF”, “LTE-M”, “NR_U”, “EUTRA_U”, “TRUSTED_N3GA”, “TRUSTED_WLAN”, “UTRA”, “GERA”. |  |  |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| periodicity |  |  |  | It identifies the time period between the start of two bursts in reference to the TSN GM. AllowedValues: see TS 29.571 [61]. |  |  |  | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |
| burstArrivalTime |  |  |  | Indicates the arrival time (in date-time format) of the data burst in reference to the TSN GM. AllowedValues: see TS 29.571 [61]. |  |  |  | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |  |  |

## 5.5	Common notifications

### 5.5.1	Alarm notifications

This clause presents a list of notifications, defined in TS 28.532 [35], that an MnS consumer may receive. The notification header attribute objectClass/objectInstance shall capture the DN of an instance of a class defined in the present document.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyNewAlarm | M | -- |
| notifyClearedAlarm | M | -- |
| notifyAckStateChanged | M | -- |
| notifyAlarmListRebuilt | M | -- |
| notifyChangedAlarm | O | -- |
| notifyCorrelatedNotificationChanged | O | -- |
| notifyChangedAlarmGeneral | O | -- |
| notifyComments | O | -- |
| notifyPotentialFaultyAlarmList | O | -- |

### 5.5.2	Configuration notifications

This clause presents a list of notifications, defined in TS 28.532 [35], that an MnS consumer may receive. The notification header attribute objectClass/objectInstance shall capture the DN of an instance of a class defined in the present document.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyMOICreation | O | -- |
| notifyMOIDeletion | O | -- |
| notifyMOIAttributeValueChanges | O | -- |
| notifyEvent | O | -- |

### 5.5.3	Threshold Crossing notifications

This clause presents a list of notifications, defined in TS 28.532 [35], that an MnS consumer may receive. The notification header attribute objectClass/objectInstance shall capture the DN of an instance of a class defined in the present document.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyThresholdCrossing | M |  |

# 6	Information model definitions for network slice NRM

## 6.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.622 [30], IOC, Top | Top |
| TS 28.622 [30], IOC, SubNetwork | SubNetwork |
| TS 28.622 [30], IOC, ManagedFunction | ManagedFunction |
| TS 28.658 [19], dataType, PLMNId | PLMNId |

## 6.2	Class diagram

### 6.2.1	Relationships

Figure 6.2.1-1: Network slice NRM fragment relationship

NOTE 1:	The <<OpenModelClass>> NetworkService and <<OpenModelClass>> VNF are defined in [40].

NOTE 2:	The target Network Service (NS) instance represents a group of VNFs and PNFs that are supporting the source network slice subnet instance.

NOTE 3:	The instance tree of this NRM fragment would not contain the instances of NetworkService and VNF. However, the NetworkSliceSubNet instances would have an attribute holding the identifiers of NetworkService instances and the ManagedFunction instance would have an attribute holding identifiers of VNF instances.

Figure 6.2.1-2: Transport EP NRM fragment relationship

### 6.2.2	Inheritance

Figure 6.2.2-1: Network slice inheritance relationship

## 6.3	Class definitions

### 6.3.1	NetworkSlice

#### 6.3.1.1	Definition

This IOC represents the properties of a network slice in a 5G network. For more information about the network slice, see 3GPP TS 28.530 [70].

#### 6.3.1.2	Attributes

The NetworkSlice IOC includes attributes inherited from SubNetwork IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |  |
| --- | --- | --- | --- | --- | --- | --- |
| operationalState | M | T | F | F | T |  |
| administrativeState | M | T | T | F | T |
| serviceProfileList | M | T | T | F | T |  |
| Attribute related to role |  |  |  |  |  |  |
| networkSliceSubnetRef | M | T | F | F | T |  |

#### 6.3.1.3	Attribute constraints

None.

#### 6.3.1.4	Notifications

The common notifications defined in subclause 6.5 are valid for this IOC, without exceptions or additions.

### 6.3.2	NetworkSliceSubnet

#### 6.3.2.1	Definition

This IOC represents the properties of a network slice subnet in a 5G network. For more information about the network slice subnet instance, see 3GPP TS 28.530 [70].

#### 6.3.2.2	Attributes

The NetworkSliceSubnet IOC includes attributes inherited from SubNetwork IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| operationalState | M | T | F | F | T |
| administrativeState | M | T | T | F | T |
| nsInfo | CM | T | F | F | T |
| sliceProfileList | M | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| managedFunctionRef | M | T | F | F | T |
| networkSliceSubnetRef | M | T | F | F | T |
| epTransportRef | O | T | T | F | T |

#### 6.3.2.3	Attribute constraints

| Name | Definition |
| --- | --- |
| nsInfo Support Qualifier | Condition: It shall be supported if the NSS instance is realized in the virtualized environment. Otherwise this attribute shall be absent. |

#### 6.3.2.4	Notifications

The common notifications defined in subclause 6.5 are valid for this IOC, without exceptions or additions.

### 6.3.3	ServiceProfile <<dataType>>

#### 6.3.3.1	Definition

This data type represents the properties of network slice related requirement that should be supported by the  NetworkSlice instance in 5G network. The network slice can be tailored based on the specific requirements adhered to SLA agreed between Network Slice Customer (NSC) and Network Slice Provider (NSP), see clause 2 of [50]. An NSP may add additional requirements not directly derived from SLA’s, associated to the NSP internal [business] goals. The GST defined by GSMA (see [50]) and the service performance requirements defined in 3GPP TS 22.261 [28] and TS 22.104 [51] are all considered as input for the network slice related requirements.

#### 6.3.3.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| serviceProfileId | M | T | F | T | T |
| sNSSAIList | M | T | T | F | T |
| pLMNIdList | M | T | T | F | T |
| maxNumberofUEs | O | T | T | F | T |
| coverageArea | O | T | T | F | T |
| latency | O | T | T | F | T |
| uEMobilityLevel | O | T | T | F | T |
| resourceSharingLevel | O | T | T | F | T |
| sST | M | T | T | F | T |
| availability | O | T | T | F | T |
| delayTolerance | O | T | T | F | T |
| deterministicComm | O | T | T | F | T |
| dLThptPerSlice | O | T | T | F | T |
| dLThptPerUE | O | T | T | F | T |
| uLThptPerSlic | O | T | T | F | T |
| uLThptPerUE | O | T | T | F | T |
| maxPktSize | O | T | T | F | T |
| maxNumberofPDUSessions | O | T | T | F | T |
| kPIMonitoring | O | T | T | F | T |
| userMgmtOpen | O | T | T | F | T |
| v2XCommModels | O | T | T | F | T |
| termDensity | O | T | T | F | T |
| activityFactor | O | T | T | F | T |
| uESpeed | O | T | T | F | T |
| jitter | O | T | T | F | T |
| survivalTime | O | T | T | F | T |
| reliability | O | T | T | F | T |
| maxDLDataVolume | O | T | T | F | T |
| maxULDataVolume | O | T | T | F | T |
| nBIoT | O | T | T | F | T |
| synchronicity | O | T | T | F | T |
| positioning | O | T | T | F | T |
| sliceSimultaneousUse | O | T | T | F | T |

NOTE:	The attributes in ServiceProfile represent mapped requirements from an NSC (e.g. an enterprise) to an NSP

#### 6.3.3.3	Attribute constraints

None.

#### 6.3.3.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.4	SliceProfile <<dataType>>

#### 6.3.4.1	Definition

This data type represents the properties of network slice subnet related requirement that should be supported by the NetworkSliceSubnet instance in a 5G network.

#### 6.3.4.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| sliceProfileId | M | T | F | T | T |
| sNSSAIList | M | T | T | F | T |
| pLMNIdList | M | T | T | F | T |
| perfReq | M | T | T | F | T |
| CNSliceSubnetProfile | CM | T | T | F | T |
| RANSliceSubnetProfile | CM | T | T | F | T |
| TopSliceSubnetProfile | CM | T | T | F | T |

#### 6.3.4.3	Attribute constraints

| Name | Definition |
| --- | --- |
| CNSliceSubnetProfile Support Qualifier | Condition: It shall be present when the slice profile for CN domain is needed. |
| RANSliceSubnetProfile Support Qualifier | Condition: It shall be present when the slice profile for RAN domain is needed. |
| tOPSliceSubnetProfile Support Qualifier | Condition: It shall be present when the slice profile is for top/root network slice subnet |

Editors Note 1: Need for specific slice profile for TN domain is FFS.

Editor's NOTE 2: Analysis on clashes/inconsistencies between perfReq attribute from SliceProfile (cf. Section 6.3.4.2) and attributes from domain-specific SliceProfiles is FFS.

Editor's NOTE 3: The common attributes of the three types of SliceProfile may be extracted out and put into the common part of the SliceProfile

Editor's NOTE 4: Whether SliceProfile is dataType or IOC is FFS.

Editor's NOTE 5: Whether RANSliceSubnetProfile is inherited from or contained by SliceProfile is FFS.

Editor's NOTE 6: Whether CNSliceSubnetProfile is inherited from or contained by SliceProfile is FFS.

Editor's NOTE 7: Whether tOPSliceSubnetProfile is inherited from or contained by SliceProfile is FFS.

Editor's NOTE 8: All the attributes of SliceProfile, CNSliceSubnetProfile, RANSliceSubnetProfile and topSliceSubnetProfile will be revisited later

#### 6.3.4.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.5	NsInfo <<dataType>>

#### 6.3.5.1	Definition

This data type represents the properties of network service information (See clause 8.3.3.2.2 of ETSI GS NFV-IFA 013 [29]) corresponding to the network slice subnet instance.

#### 6.3.5.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nSInstanceId | M | T | F | F | T |
| nsName | O | T | F | F | T |
| description | O | T | F | F | T |

#### 6.3.5.3	Attribute constraints

None.

#### 6.3.5.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.6	ServAttrCom <<dataType>>

#### 6.3.6.1	Definition

This data type represents the common properties of service requirement related attributes (see GSMA NG.116 [50] corresponding to Attribute categories, tagging and exposure).

#### 6.3.6.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| category | M | T | F | F | T |
| tagging | CM | T | F | F | T |
| exposure | M | T | F | F | T |

#### 6.3.6.3	Attribute constraints

| Name | Definition |
| --- | --- |
| tagging Support Qualifier | Condition: It shall be supported if the category is character. Otherwise this attribute shall be absent. |

#### 6.3.6.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.7	DelayTolerance<<dataType>>

#### 6.3.7.1	Definition

This data type represents the delay tolerance (See Clause 3.4.3 of GSMA NG.116 [50]).

#### 6.3.7.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | M | T | F | F | T |
| support | M | T | F | F | T |

#### 6.3.7.3	Attribute constraints

None.

#### 6.3.7.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.7	DeterminComm <<dataType>>

#### 6.3.7.1	Definition

This data type represents the properties of the deterministic communication for periodic user traffic. Periodic traffic refers to the type of traffic with periodic transmissions.

#### 6.3.7.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | M | T | F | F | T |
| availability | M | T | F | F | T |
| periodicityList | M | T | T | F | T |

#### 6.3.7.3	Attribute constraints

None.

#### 6.3.7.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.8	DLThpt<<dataType>>

#### 6.3.8.1	Definition

This data type represents the downlink throughput per slice or per UE (See Clause 3.4.5 and 3.4.6 of GSMA NG.116 [50]).

#### 6.3.8.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | M | T | F | F | T |
| guaThpt | M | T | F | F | T |
| maxThpt | C | T | F | F | T |

#### 6.3.8.3	Attribute constraints

None.

#### 6.3.8.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.9	ULThpt<<dataType>>

#### 6.3.9.1	Definition

This data type represents the uplink throughput per slice or per UE (See Clause 3.4.31 and 3.4.32 of GSMA NG.116 [50]).

#### 6.3.9.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | M | T | F | F | T |
| guaThpt | O | T | F | F | T |
| maxThpt | O | T | F | F | T |

#### 6.3.9.3	Attribute constraints

None.

#### 6.3.9.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.10	MaxPktSize <<dataType>>

#### 6.3.10.1	Definition

This data type represents the maximum packet size (See Clause 3.4.11 of GSMA NG.116 [50]).

#### 6.3.10.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | M | T | F | F | T |
| maxSize | M | T | F | F | T |

#### 6.3.10.3	Attribute constraints

None.

#### 6.3.10.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.11	MaxNumberofPDUSessions <<dataType>>

#### 6.3.11.1	Definition

This data type represents the maximum number of concurrent PDU sessions supported by the network slice (see clause 3.4.16 of GSMA NG.116 [50]).

#### 6.3.11.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | M | T | F | F | T |
| nOofPDUSessions | M | T | F | F | T |

#### 6.3.11.3	Attribute constraints

None.

#### 6.3.11.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.12	Void

### 6.3.13	KPIMonitoring <<dataType>>

#### 6.3.13.1	Definition

This data type represents performance monitoring (See Clause 3.4.17 of GSMA NG.116 [50]).

#### 6.3.13.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | M | T | F | F | T |
| kPIList | M | T | F | F | T |

#### 6.3.13.3	Attribute constraints

None.

#### 6.3.13.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.14	UserMgmtOpen<<dataType>>

#### 6.3.14.1	Definition

This data type represents User management openness (See Clause 3.4.33 of GSMA NG.116 [50]).

#### 6.3.14.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | M | T | F | F | T |
| support | M | T | F | F | T |

#### 6.3.14.3	Attribute constraints

None.

#### 6.3.14.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.15	V2XCommMode<<dataType>>

#### 6.3.15.1	Definition

This data type represents V2X communication mode (See Clause 3.4.35 of GSMA NG.116 [50]).

#### 6.3.15.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | M | T | F | F | T |
| v2XMode | M | T | F | F | T |

#### 6.3.15.3	Attribute constraints

None.

#### 6.3.15.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.16	TermDensity<<dataType>>

#### 6.3.16.1	Definition

This data type represents Terminal density (See Clause 3.4.30 of GSMA NG.116 [50]).

#### 6.3.16.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | M | T | F | F | T |
| density | M | T | F | F | T |

#### 6.3.16.3	Attribute constraints

None.

#### 6.3.16.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.17	EP_Transport

#### 6.3.17.1	Definition

This IOC represents the logical transport interface or endpoint which including transport level information, e.g. transport address, reachability information and QoS profiles, etc.

The IOC is inherited from Top IOC.

#### 6.3.17.2	Attributes

The EP_Transport IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| ipAddress | M | T | F | F | T |
| logicInterfaceId | M | T | T | F | T |
| nextHopInfo | O | T | F | F | T |
| qosProfile | O | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| epApplicationRef | M | T | T | F | T |

#### 6.3.17.3	Attribute constraints

None.

#### 6.3.17.4	Notifications

The common notifications defined in subclause 6.5 are valid for this IOC, without exceptions or additions.

### 6.3.18	EP_Application <<ProxyClass>>

#### 6.3.18.1	Definition

This represents  <<IOC>>EP_N3 or <<IOC>>EP_NgU.

#### 6.3.18.2	Attributes

See that defined in <<IOC>>EP_N3 or <<IOC>>EP_NgU.

#### 6.3.18.3	Attribute constraints

See respective IOCs.

#### 6.3.18.4	Notifications

See respective IOCs.

### 6.3.19	NBIoT <<dataType>>

#### 6.3.19.1	Definition

This data type represents NB-IoT Support (see clause 3.4.14 of GSMA NG.116 [50]).

#### 6.3.19.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | M | T | F | F | T |
| support | M | T | F | F | T |

#### 6.3.19.3	Attribute constraints

None.

#### 6.3.19.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.20	DLThptSliceSubnet<<dataType>>

#### 6.3.20.1	Definition

This data type represents the downlink throughput per slice subnet or per UE.

#### 6.3.20.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| guaThpt | M | T | F | F | T |
| maxThpt | C | T | F | F | T |

#### 6.3.20.3	Attribute constraints

None.

#### 6.3.20.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.21	ULThptSliceSubnet<<dataType>>

#### 6.3.21.1	Definition

This data type represents the uplink throughput per slice subnet or per UE.

#### 6.3.21.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| guaThpt | M | T | F | F | T |
| maxThpt | C | T | F | F | T |

#### 6.3.21.3	Attribute constraints

None.

#### 6.3.21.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.22	CNSliceSubnetProfile<<dataType>>

#### 6.3.22.1	Definition

This data type represents the requirements for CN slice profile.

Editor's NOTE: Whether CNSliceSubnetProfile is an IOC or dataType is FFS.

#### 6.3.22.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| maxNumberofUEs | O | T | T | F | T |
| latency | O | T | T | F | T |
| dLThptPerSliceSubnet | O | T | T | F | T |
| dLThptPerUEPerSubnet | O | T | T | F | T |
| uLThptPerSliceSubnet | O | T | T | F | T |
| uLThptPerUEPerSubnet | O | T | T | F | T |
| maxNumberOfPDUSessions | O | T | T | F | T |
| coverageAreaTAList | O | T | T | F | T |
| resourceSharingLevel | O | T | T | F | T |
| maxPktSize | O | T | T | F | T |
| sliceSimultaneousUse | O | T | T | F | T |
| delayTolerance | O | T | T | F | T |

#### 6.3.22.3	Attribute constraints

None.

#### 6.3.22.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.23	RANSliceSubnetProfile<<dataType>>

#### 6.3.23.1	Definition

This data type represents the requirements for RAN slice profile.

Editor's NOTE 1: Whether the attributes of RANSliceSubnetProfile need to be modelled by one IOC or more than one IOC is FFS.

Editor's NOTE 2: Whether RANSliceSubnetProfile is an IOC or dataType is FFS.

#### 6.3.23.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| coverageAreaTAList | O | T | T | F | T |
| uEMobilityLevel | O | T | T | F | T |
| resourceSharingLevel | O | T | T | F | T |
| maxNumberofUEs | O | T | T | F | T |
| activityFactor | O | T | T | F | T |
| dLThptPerUEPerSubnet | O | T | T | F | T |
| uLThptPerUEPerSubnet | O | T | T | F | T |
| uESpeed | O | T | T | F | T |
| reliability | O | T | T | F | T |
| serviceType | O | T | T | F | T |
| latency | O | T | T | F | T |
| delayTolerance | O | T | T | F | T |
| sliceSimultaneousUse | O | T | T | F | T |
| maxPktSize | O | T | T | F | T |

#### 6.3.23.3	Attribute constraints

None.

#### 6.3.23.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.24	TopSliceSubnetProfile<<dataType>>

#### 6.3.24.1	Definition

This data type represents the requirements for the top slice associated with the network slice.

Editor's NOTE: Whether TopSliceSubnetProfile is an IOC or dataType is FFS.

#### 6.3.24.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| coverageArea | O | T | T | F | T |
| latency | O | T | T | F | T |
| maxNumberofUEs | O | T | T | F | T |
| dLThptPerSliceSubnet | O | T | T | F | T |
| dLThptPerUEPerSubnet | O | T | T | F | T |
| uLThptPerSliceSubnet | O | T | T | F | T |
| uLThptPerUEPerSubnet | O | T | T | F | T |
| maxPktSize | O | T | T | F | T |
| maxNumberOfPDUSessions | O | T | T | F | T |
| sliceSimultaneousUse | O | T | T | F | T |
| delayTolerance | O | T | T | F | T |

#### 6.3.24.3	Attribute constraints

None.

#### 6.3.24.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.25	Positioning <<dataType>>

#### 6.3.25.1	Definition

This data type represents positioning support (see clause 3.4.20 of GSMA NG.116 [50]).

#### 6.3.25.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | M | T | F | F | T |
| availability | O | T | F | F | T |
| predictionfrequency | O | T | T | F | T |
| accuracy | O | T | T | F | T |

#### 6.3.25.3	Attribute constraints

None.

#### 6.3.25.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.26	Synchronicity <<dataType>>

#### 6.3.26.1	Definition

This data type represents synchronicity support (see clause 3.4.29 of GSMA NG.116 [50]).

#### 6.3.26.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| servAttrCom | M | T | F | F | T |
| availability | O | T | F | F | T |
| accuracy | O | T | T | F | T |

#### 6.3.26.3	Attribute constraints

None.

#### 6.3.26.4	Notifications

The subclause 6.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

## 6.4	Attribute definition

### 6.4.1	Attribute properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| availability | This parameter specifies the communication service availability requirement, expressed as a percentage. The communication service availability is defined in clause 3.1 of TS 22.261 [28]. | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: True |
| serviceProfileId | A unique identifier of property of network slice related requirement should be supported by the network slice. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| sliceProfileId | A unique identifier of the property of network slice subnet related requirement should be supported by the network slice subnet. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| operationalState | It indicates the operational state of the network slice or the network slice subnet. It describes whether or not the resource is physically installed and working. allowedValues: "ENABLED", "DISABLED". The meaning of these values is as defined in 3GPP TS 28.625 [17] and ITU-T X.731 [18]. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| administrativeState | It indicates the administrative state of the network slice or the network slice subnet. It describes the permission to use or prohibition against using the managed object instance, imposed through the OAM services. allowedValues: “LOCKED”, “UNLOCKED”, SHUTTINGDOWN” The meaning of these values is as defined in 3GPP TS 28.625 [17] and ITU-T X.731 [18]. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: LOCKED allowedValues: N/A isNullable: False |
| nsInfo | This attribute contains the NsInfo of the NS instance corresponding to the network slice subnet instance. The NsInfo is described in clause 8.3.3.2.2 of ETSI GS NFV-IFA 013 [29]. | type: NsInfo multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: No default value isNullable: True |
| nSInstanceId | This attribute specifies the identifier of NS instance corresponding to the network slice subnet instance. See clause 8.3.3.2.2 of ETSI GS NFV-IFA 013 [29]. | type: String multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: No default value isNullable: True |
| nsName | This attribute specifies the name of NS instance corresponding to the network slice subnet instance. See clause 8.3.3.2.2 of ETSI GS NFV-IFA 013 [29]. | type: String multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: No default value isNullable: True |
| description | This attribute specifies the description of NS instance corresponding to the network slice subnet instance. See clause 8.3.3.2.2 of ETSI GS NFV-IFA 013 [29]. | type: String multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: No default value isNullable: True |
| category | This attribute specifies the category of a service requirement/attribute of GST (see GSMA NG.116 [50]). allowedValues: character, scalability | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| tagging | This attribute specifies the tagging of a service requirement/attribute of GST in character category (see GSMA NG.116 [50]). allowedValues: performance, function, operation | type: ENUM multiplicity: 1…3 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| exposure | This attribute specifies exposure mode of a service requirement/attribute of GST (see GSMA NG.116 [50]). allowedValues: API, KPI | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| sNSSAIList | This parameter specifies the S-NSSAI list to be supported by the network slice new to be created or the existing network slice to be re-used. sNSSAList is defined in subclause 4.4.1 |  |
| perfReq | This parameter specifies the requirements to the network slice subnet in terms of the scenarios defined in the TS 22.261 [28] and TS 22.104 [51], i.e. the "performance requirements for high data rate and traffic density scenarios" in TS 22.261 [28], "periodic deterministic communication, aperiodic deterministic communication, non-deterministic communication, and mixed traffic" in TS 22.104 [51]. It is a structure containing the following elements: - list of perfReq Depending on the sST value, the list of perfReq will be - list of eMBBPerfReq or - list of uRLLCPerfReq or - list of mIoTPerfReq NOTE 1: the list of mIoTPerfReq is not addressed in the present document. allowedValues: - list of eMBBPerfReq is a list of entries where an entry identifies the performance requirements to the network slice subnet in terms of the scenarios defined in the Table 7.1-1 of TS 22.261 [28]. An entry has the following attributes: expDataRateDL (Integer), expDataRateUL (Integer), areaTrafficCapDL (Integer), areaTrafficCapUL (Integer), overallUserDensity (Integer), activityFactor (Integer), (see table 7.1-1 of TS 22.261 [28]). - list of uRLLCPerfReq is a list of entries where an entry identifies the performance requirements to the network slice subnet in terms of the scenarios defined in clauses 5.2 through 5.5 of TS 22.104 [51]. An entry has the following attributes: cSAvailabilityTarget (Float), cSReliabilityMeanTime (String), , expDataRate (Integer), msgSizeByte (String), transferIntervalTarget (String), survivalTime (String), , , (see table 5.2-1, table 5.3-1, table 5.4-1 and table 5.5-1 of TS 22.104 [51]). NOTE 2: Limitation on attribute values in SliceProfile is not addressed in the present document. NOTE 3: The attributes inside perfReq here need further breaking down to define requirements for each subnetwork under different SST values. | type: PerfReq multiplicity: *1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| maxNumberofUEs | An attribute specifies the maximum number of UEs may simultaneously access the network slice or network slice subnet instance. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| coverageAreaTAList | An attribute specifies a list of Tracking Areas for the network slice . allowedValues: Legacy TAC and Extended TAC are defined in clause 9.3.3.10 of TS 38.413 [5]. | type: Integer multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| latency | An attribute specifies the packet transmission latency (millisecond) through the RAN, CN, and TN part of 5G network and is used to evaluate utilization performance of the end-to-end network slice. See clause 6.3.1 of 28.554 [27]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| topSliceSubnetProfile.latency | An attribute specifies the packet transmission latency (millisecond) through all domains of the network slice and is used to evaluate utilization performance of the end-to-end network slice. See clause 6.3.1 of 28.554 [27]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| CNSliceSubnetProfile.latency | An attribute specifies the packet transmission latency (millisecond) through CN domain of the network slice and is used to evaluate the delay in CN domain, e.g. time between received UL/DL packet on N3/N6 interface of UPF and successfully sent out the packet on N6/N3 interface. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| RANSliceSubnetProfile.latency | An attribute specifies the packet transmission latency (millisecond) through RAN domain of the network slice and is used to evaluate the delay in RAN domain, e.g. time between received UL/DL packet on air interface/NgU of gNB and successfully sent out the packet on NgU/air interface of the gNB. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| uEMobilityLevel | An attribute specifies the mobility level of UE accessing the network slice. See 6.2.1 of TS 22.261 [28]. allowedValues: stationary, nomadic, restricted mobility, fully mobility. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: True |
| serviceProfile.resourceSharingLevel | An attribute specifies whether the resources to be allocated to the network slice may be shared with another network slice(s). allowedValues: shared, non-shared. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: Yes isNullable: True |
| sliceProfile.resourceSharingLevel | An attribute specifies whether the resources to be allocated to the network slice subnet may be shared with another network slice subnet(s). allowedValues: shared, non-shared. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: Yes isNullable: True |
| serviceProfileList | An attribute specifies a list of ServiceProfile (see clause 6.3.3) supported by the network slice | type: ServiceProfile multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| sliceProfileList | An attribute specifies a list of SliceProfile (see clause 6.3.4) supported by the network slice subnet | type: SliceProfile multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| sST | This parameter specifies the slice/service type in a ServiceProfile to be supported by a network slice. See clause 5.15.2 of 3GPP TS 23.501 [2]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| delayTolerance | An attribute specifies the properties of service delivery flexibility, especially for the vertical services that are not chasing a high system performance. See clause 4.3 of TS 22.104 [51]. | type: DelayTolerance multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| DelayTolerance.support | An attribute specifies whether or not the network slice supports service delivery flexibility, especially for the vertical services that are not chasing a high system performance. allowedValues: "NOT SUPPORTED", "SUPPORTED". | type: <<enumeration>> multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| deterministicComm | An attribute specifies the properties of the deterministic communication for periodic user traffic, see clause 4.3 of TS 22.104 [51]. | type: <<DeterminComm>> multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| DeterminComm.availability | An attribute specifies whether or not the network slice supports deterministic communication for period user traffic. allowedValues: "NOT SUPPORTED", "SUPPORTED". | type: <<enumeration>> multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| DeterminComm.periodicityList | An attribute specifies a list of periodicities supported by the network slice for deterministic communication. | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| dLThptPerSlice | This attribute defines achievable data rate of the network slice in downlink that is available ubiquitously across the coverage area of the slice, refer NG.116 [50]. | type: DLThpt multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| dLThptPerSliceSubnet | This attribute defines achievable data rate of the network slice subnet in downlink that is available ubiquitously across the coverage area of the slice. | type: DLThptSliceSubnet multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| dLThptPerUEPerSubnet | This attribute defines data rate supported by the network slice subnet per UE. | type: DLThptSliceSubnet multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| dLThptPerUE | This attribute defines data rate supported by the network slice per UE, refer NG.116 [50]. | type: DLThpt multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| guaThpt | This attribute describes the guaranteed data rate. | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| maxThpt | This attribute describes the maximum data rate. | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| uLThptPerSlice | This attribute defines achievable data rate of the network slice in uplink that is available ubiquitously across the coverage area of the slice, refer NG.116 [50]. | type: ULThpt multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| uLThptPerUE | This attribute defines data rate supported by the network slice per UE, refer NG.116 [50]. | type: ULThpt multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| uLThptPerSliceSubnet | This attribute defines achievable data rate of the network slice subnet in uplink that is available ubiquitously across the coverage area of the slice. | type: ULThptSliceSubnet multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| uLThptPerUEPerSubnet | This attribute defines data rate supported by the network slice subnet per UE. | type: ULThptSliceSubnet multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| maxPktSize | This parameter specifies the maximum packet size supported by the network slice or the network slice subnet, refer NG.116 [50]. | type: MaxPktSize multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| MaxPktSize.maxsize | This parameter specifies the maximum packet size supported by the network slice, refer NG.116 [50]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| maxNumberofPDUSessions | This parameter defines the maximum number of concurrent PDU sessions supported by the network slice, refer NG.116 [50]. | type: MaxNumberofPDUSessions multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| MaxNumberofPDUSessions.nOofPDUSessions | This parameter defines the maximum number of concurrent PDU sessions supported by the network slice, refer NG.116 [50]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| kPIMonitoring | An attribute specifies the name list of KQIs and KPIs available for performance monitoring. | type: KPIMonitoring multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| KPIMonitoring. kPIList | An attribute specifies the name list of KQIs and KPIs available for performance monitoring. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| nBIoT | An attribute specifies whether NB-IoT is supported in the RAN in the network slice, see NG.116 [50]. | type: NBIoT multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| NBIoT.support | An attribute specifies whether NB-IoT is supported in the RAN in the network slice, see NG.116 [50]. allowedValues: "NOT SUPPORTED", "SUPPORTED". | type: <<enumeration>> multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| synchronicity | An attribute specifies whether synchronicity of communication devices is supported, Two cases are most important in this context, see clause 3.4.29 of NG.116 [50]: - Synchronicity between a base station and a mobile device and - Synchronicity between mobile devices. | type: Synchronicity multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| Synchronicity.availability | An attribute specifies whether synchronicity of communication devices is supported, see NG.116 [50]. allowedValues: "NOT SUPPORTED", "BETWEEN BS AND UE", "BETWEEN BS AND UE & UE AND UE". | type: <<enumeration>> multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| Synchronicity.accuracy | An attribute specifies the accuracy of the synchronicity, see NG.116 [50]. | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| userMgmtOpen | An attribute specifies whether or not the network slice supports the capability for the NSC to manage their users or groups of users’ network services and corresponding requirements. | type: UserMgmtOpen multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| UserMgmtOpen.support | An attribute specifies whether or not the network slice supports the capability for the NSC to manage their users or groups of users’ network services and corresponding requirements. allowedValues: "NOT SUPPORTED", "SUPPORTED". | type: <<enumeration>> multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| v2XCommModels | An attribute specifies whether or not the V2X communication mode is supported by the network slice. | type: V2XCommMode multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| V2XCommMode.v2XMode | An attribute specifies whether or not the V2X communication mode is supported by the network slice. allowedValues: "NOT SUPPORTED", "SUPPORTED BY NR". | type: <<enumeration>> multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| coverageArea | An attribute specifies the coverage area of the network slice, i.e. the geographic region where a 3GPP communication service is accessible, see Table 7.1-1 of TS 22.261 [28]) and NG.116 [50]. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| termDensity | An attribute specifies the overall user density over the coverage area of the network slice. See Table 7.1-1 of TS 22.261 [28]). | type: TermDensity multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| TermDensity.density | An attribute specifies the overall user density over the coverage area of the network slice. See Table 7.1-1 of TS 22.261 [28]). | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| positioning | An attribute specifies whether the network slice provides geo-localization methods or supporting methods, see clause 3.4.20 of NG.116 [50]. | type: Positioning multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| Positioning.availability | An attribute specifies if this attribute is provided by the network slice and contains a list of positioning methods provided by the slice. If the list is empty this attribute is not available in the network slice and the other parameters might be ignored, see NG.116 [50]. Comma separated multiple values are allowed: CIDE-CID (LTE and NR), OTDOA (LTE and NR), RF fingerprinting, AECID, Hybrid positioning, NET-RTK. | type: ENUM multiplicity: 1..6 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| Positioning.predictionfrequency | An attribute specifies how often location information is provided. This parameter simply defines how often the customer is allowed to request location information. This is not related to the time it takes to determine the location, which is a characteristic of the positioning method, see NG.116 [50]. allowedValues: "PERSEC", "PERMIN", "PERHOUR". | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| Positioning.accuracy | An attribute specifies the accuracy of the location information. Accuracy depends on the respective positioning solution applied in the network slice, see NG.116 [50]. | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| activityFactor | An attribute specifies the percentage value of the amount of simultaneous active UEs to the total number of UEs where active means the UEs are exchanging data with the network. See Table 7.1-1 of TS 22.261 [28]). | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| uESpeed | An attribute specifies the maximum speed (in km/hour) supported by the network slice or network slice subnet at which a defined QoS can be achieved. See Table 7.1-1 of TS 22.261 [28]). | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| jitter | An attribute specifies the deviation from the desired value to the actual value when assessing time parameters, see clause C.4.1 of TS 22.104 [51]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| survivalTime | An attribute specifies the time that an application consuming a communication service may continue without an anticipated message. See clause 5 of TS 22.104 [51]). | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| reliability | An attribute specifies in the context of network layer packet transmissions, percentage value of the amount of sent network layer packets successfully delivered to a given system entity within the time constraint required by the targeted service, divided by the total number of sent network layer packets, see TS 22.261 [28] and TS 22.104 [51]. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| NetworkSlice.networkSliceSubnetRef | This holds a DN of NetworkSliceSubnet relating to the NetworkSlice instance. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NetworkSliceSubnet.networkSliceSubnetRef | This holds a list of DN of constituent NetworkSliceSubnet supporting NetworkSliceSubnet instance | type: DN multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| managedFunctionRef | This holds a list of DN of ManagedFunction instances supporting the NetworkSliceSubnet instance. | type: DN multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| ipAddress | This parameter specifies the IP address assigned to a logical transport interface/endpoint. It can be an IPv4 address (See RFC 791 [37]) or an IPv6 address (See RFC 2373 [38]). See note 1 | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| logicInterfaceId | This parameter specifies the identify of a logical transport interface. It could be VLAN ID (See IEEE 802.1Q [39]), MPLS Tag or Segment ID. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nextHopInfoList | This parameter is used to identify ingress transport node. Each node can be identified by any of combination of IP address of next-hop router of transport network, system name, port name, IP management address of transport nodes. | type: String multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| qosProfileRefList | This parameter specifies reference to QoS Profile for a logical transport interface. A QoS profile includes a set of parameters which are locally provisioned on both sides of a logical transport interface. | type: String multiplicity: * isOrdered: N/A isUnique: True defaultValue: None isNullable: True |
| maxDLDataVolume | An attribute specifies the maximum DL PDCP data volume supported by the network slice instance (performance measurement definition see in TS 28.552[69]). The unit is MByte/day. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| maxULDataVolume | An attribute specifies the maximum UL PDCP data volume supported by the network slice instance (performance measurement definition see in TS 28.552[69]). The unit is MByte/day. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| serviceType | An attribute specifies the standardized network slice type. allowedValues: eMBB, URLLC, MIoT, V2X. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: True |
| epApplicationRef | This parameter specifies a list of application level EPs associated with the logical transport interface. See note 2. | type: DN multiplicity: * isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| epTransportRef | This parameter specifies a list of transport level EPs associated with the application level EP (i.e. EP_N3 or EP_NgU) or network slice subnet. | type: DN multiplicity: * isOrdered: N/A isUnique: True defaultValue: None isNullable: True |
| sliceSimultaneousUse | This attribute describes whether a network slice can be simultaneously used by a device together with other network slices and if so, with which other classes of network slices. allowedValues: “0”, “1”, “2”, “3”, “4”. “0”: Can be used with any network slice “1”: Can be used with network slices with same SST value “2”: Can be used with any network slice with same SD value “3”: Cannot be used with another network slice “4”: Cannot be used by a UE in a specific location | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| NOTE 1: There is no direct relationship between localAddress/remoteAddress in EP_RP and ipAddress in EP_transport. While the localAddress/remoteAddress in EP_RP could be exchanged as part of signalling between GTP-u tunnel end points, ipAddress in EP_transport is used for transport routing. NOTE 2: Application level EP represents EP_RP defined in TS 28.622 (see [30]). e.g. including EP_NgC, EP_N3, etc... |  |  |

## 6.5	Common notifications

### 6.5.1	Alarm notifications

This clause presents a list of notifications, defined in TS 28.532 [35], that an MnS consumer may receive. The notification header attribute objectClass/objectInstance shall capture the DN of an instance of a class defined in the present document.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyNewAlarm | M | -- |
| notifyClearedAlarm | M | -- |
| notifyAckStateChanged | M | -- |
| notifyAlarmListRebuilt | M | -- |
| notifyChangedAlarm | O | -- |
| notifyCorrelatedNotificationChanged | O | -- |
| notifyChangedAlarmGeneral | O | -- |
| notifyComments | O | -- |
| notifyPotentialFaultyAlarmList | O | -- |

### 6.5.2	Configuration notifications

This clause presents a list of notifications, defined in TS 28.532 [35], that an MnS consumer may receive. The notification header attribute objectClass/objectInstance shall capture the DN of an instance of a class defined in the present document.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyMOICreation | O | -- |
| notifyMOIDeletion | O | -- |
| notifyMOIAttributeValueChanges | O | -- |
| notifyEvent | O | -- |

### 6.5.3	Threshold Crossing notifications

This clause presents a list of notifications, defined in TS 28.532 [35], that an MnS consumer may receive. The notification header attribute objectClass/objectInstance shall capture the DN of an instance of a class defined in the present document.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyThresholdCrossing | M |  |

# 7	Solution Set (SS)

The present document defines the following NRM Solution Set definitions for NR and NG-RAN:

-	JSON based 3GPP NR and NG-RAN NRM Solution Set (Annex D).

-	YANG based 3GPP NR and NG-RAN NRM Solution Set (Annex E).

The present document defines the following NRM Solution Set definitions for 5GC:

-	JSON based 3GPP 5GC NRM Solution Set (Annex G).

-	YANG based 3GPP 5GC NRM Solution Set (Annex H).

The present document defines the following NRM Solution Set definitions for network slice and network slice subnet:

-	JSON based 3GPP Network Slice NRM Solution Set (Annex J).

-	YANG based 3GPP Network Slice NRM Solution Set (Annex O)

Annex A (normative):
Cell state handling

# A.1	Relation between the administrative state and the "Pre-operation state of the gNB-DU Cell"

The administrative state indicates the permission to use or prohibition against using the cell, imposed through the OAM services. The administrative state has three values: "LOCKED", "SHUTTING DOWN" or "UNLOCKED"

The meanings of these values are defined in ITU-T Recommendation X.731 [18].

The relation between the administrative state and the "Pre-operation state of the gNB-DU Cell" is defined in subclause 8.5 of TS 38.401 [4]. See below an extract from subclause 8.5 of TS 38.401 [4] on the F1 startup and cell activation.

If the operationalState is "ENABLED" (i.e. the resource is physically installed and working) and if the administrativeState is "UNLOCKED", the step "0: Pre-operational state" will exit and the step "1: F1 Setup Request" will be executed."

8.5	F1 Startup and cells activation

This function allows to setup the F1 interface between a gNB-DU and a gNB-CU and it allows to activate the gNB-DU cells.

Figure 8.5-1: F1 startup and cell activation

# A.2	Combined state diagram for gNB cell

This is the Combined state diagram for gNB cell.

Figure A.2-1: Combined gNB cell state diagram

The gNB-DU maintains cell states. The following table is the gNB cell state transition table.

In 3-split and 2-split deployment scenarios, the interactions between gNB-CU and gNB-DU are standardized. The interactions specified under the column "The state transition events and actions" of "The gNB Cell state transition table" below shall be present for the state transition.

In the non-split deployment scenarios, the interactions between gNB-CU and gNB-DU are not standardized. The interactions between gNB-CU and gNB-DU specified under the column "The state transition events and actions" of "The gNB Cell state transition table" can be replaced by other means that is not standardized.

Table A.2-1: The gNB Cell state transition table

| Transition number | The state transition event and actions |
| --- | --- |
| 1 | Event: Receive request to unlock. Action: None. |
| 2 | Event: Receive request to lock. Action: None. |
| 2a | Event: Receive request to lock Action: Send to gNB-CU the "gNB-DU Configuration Update message" with served cell to delete. |
| 3 | Event: When the required cell resource is physically installed and working. Action: none. |
| 4 | Event: When the required cell resource is not physically installed or is not working. Action: Send to gNB-CU the "gNB-DU Configuration update message" with cell to delete. |
| 4a | Event: When the required cell resource is physically uninstalled or is not working. Action: Send to gNB-CU the "GNB-DU Configuration Update message" with served cell to delete. |
| 5 | Event: Receive from gNB-CU the "F1 Setup Response message" (identifying the cell to be activated). The cell is activated successfully. Actions: Do nothing or send gNB-CU the "gNB-DU Configuration Update message" with Cell stated as active' ----- or ----- Event: Receive from gNB-CU the "gNB-CU Configuration Update message" (identifying cell to be activated e.g., in case that the cell was not activated using the "F1 Setup Response message"). Actions: The cell is activated successfully. Send to gNB-CU the "gNB-CU Configuration Update Response" to confirm the cell is in active state. ----- or ----- Event: Receive from gNB-CU the "gNB-DU Configuration Update Acknowledge message" (identifying cell to be activated e.g., in case that the cell was not activated using the "F1 Setup Response message") and the cell is activated successfully Actions: Do nothing. |
| 6 | Event: Receive from gNB-CU the "gNB-CU Configuration Update message" and responds with gNB-CU Configuration Update Acknowledge messages. Actions: Respond with gNB-CU Configuration Update Acknowledge messages. ----- or ----- Event: Event: DU experiences an internal failure and decided to place the cell into inactive state. Actions: Send to gNB-CU the "gNB-DU Cell status Update message" |
| 7 | Event: Send to gNB-CU the "F1 Setup request" (identifying the cell that is configured and ready to be activated). Actions: none. ----- or ----- Send to gNB-CU the "gNB-DU Configuration Update message" with the served cell to add. Actions: none. |
| 8 | Event: Sends to gNB-CU the "gNB-DU Configuration Update message" with served cell to delete. Receive response from gNB-CU the "gNB-DU Configuration Update Acknowledge message". Actions: None. |
| 9 | Event: Receive request to shut down. Actions: None. |
| 10 | Event: Last user quit. Actions: Send to gNB-CU the "GNB-DU Configuration Update message" with served cell to delete. |
| 11 | Event: When a cell is created and is configured. Actions: None |
| 12 | Event: When a cell is deleted. Action: None. |

Annex B (normative): 
NSI and NSSI state handling

# B.1	NSI state handling

A NetworkSlice instance (NSI) is a logical object in the management system that represents a complex grouping of resources that may be in various states. At any time, the management system needs to know the state of an NSI.

The ITU-T X.731 [18], to which [17] refers, has defined the inter-relation between the administrative state and operational state of systems in general.

Figure B.1: Combined NSI state diagram

The interactions specified under the column "The state transition events and actions" of "NSI state transition table" below shall be present for the state transition.

Table B.1: The NSI state transition table

| Trigger number | The state transition events and actions |
| --- | --- |
| 0 | Operation allocateNsi results in the creation of NSI. The administrative state is set to LOCKED and operationalState is set to DISABLED -- or – CM operation creates NSI. The administrative state is set to LOCKED and operationalState is set to DISABLED |
| 1 | CM operation sets administrative state to UNLOCKED. |
| 2 | The last user of the NSI stops using the NSI |
| 2a | CM operation sets administrative state to SHUTTING DOWN |
| 2b | The last user of the NSInetwork slice stops using the NSInetwork slice |
| 3 | The related NSSI (identified by NetworkSlice.networkSliceSubnetRef) changes state to UNLOCKED and ENABLED. |
| 4 | The related NSSI (identified by NetworkSlice.networkSliceSubnetRef) changes state to LOCKED -- or – The related NSSI (identified by NetworkSlice.networkSliceSubnetRef) changes state to DISABLED |
| 5 | Operation deallocateNsi results in the deletion of NSI -- or – CM operation deletes NSI |

# B.2	State handling of NSSI

A NetworkSliceSubnet instance (NSSI) is a logical object in the management system that represents a complex grouping of resources that may be in various states. At any time the management system needs to know the state of an NSSI.

The ITU-T X.731 [18], to which [17] refers, has defined the inter-relation between the administrative state and operational state of systems in general.

Figure B.2.1: Combined NSSI state diagram

The interactions specified under the column "The state transition events and actions" of "NSSI state transition table" below shall be present for the state transition.

Table B.2.1: The NSSI state transition table

| Trigger number | The state transition events and actions |
| --- | --- |
| 0 | Operation allocateNssi results in the creation of NSSI. The administrative state is set to LOCKED and operationalState is set to DISABLED -- or – CM operation creates NSSI. The administrative state is set to LOCKED and operationalState is set to DISABLED |
| 1 | CM operation sets administrative state to UNLOCKED. |
| 2 | CM operation sets administrative state to LOCKED |
| 2a | CM operation sets administrative state to SHUTTING DOWN |
| 2b | The last user of the NSSInetwork slice subnet stops using the NSSInetwork slice subnet |
| 3 | All constituent NSSIs (identified by NetworkSliceSubnet.networkSliceSubnetRef) change state to UNLOCKED and ENABLED. |
| 4 | At least one constituent NSSI (identified by NetworkSliceSubnet.networkSliceSubnetRef) changes state to LOCKED -- or – At least one constituent NSSI (identified by NetworkSliceSubnet.networkSliceSubnetRef) changes state to DISABLED |
| 5 | Operation deallocateNssi results in the deletion of NSSI -- or – CM operation deletes NSSI. |

Annex C (normative):
Void

Annex D (normative):
OpenAPI definition of the NR NRM

# D.1	General

This annex contains the OpenAPI definition of the NR NRM in YAML format.

The Information Service (IS) of the NR NRM is defined in clause 4.

Mapping rules to produce the OpenAPI definition based on the IS are defined in 3GPP TS 32.160 [14].

# D.2	Void

# D.3	Void

# D.4	Solution Set (SS) definitions

## D.4.1	Void

## D.4.2	Void

## D.4.3	OpenAPI document "nrNrm.yaml"

openapi: 3.0.1

info:

title: NR NRM

version: 17.2.0

description: >-

OAS 3.0.1 specification of the NR NRM

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.541 V17.2.0; 5G NRM, NR NRM

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.541/

paths: {}

components:

schemas:

#-------- Definition of types-----------------------------------------------------

GnbId:

type: string

GnbIdLength:

type: integer

minimum: 22

maximum: 32

GnbName:

type: string

maxLength: 150

GnbDuId:

type: number

minimum: 0

maximum: 68719476735

GnbCuUpId:

type: number

minimum: 0

maximum: 68719476735

Sst:

type: integer

maximum: 255

Snssai:

type: object

properties:

sst:

$ref: '#/components/schemas/Sst'

sd:

type: string

SnssaiList:

type: array

items:

$ref: '#/components/schemas/Snssai'

Mnc:

type: string

pattern: '[0-9]{3}|[0-9]{2}'

PlmnId:

type: object

properties:

mcc:

$ref: 'genericNrm.yaml#/components/schemas/Mcc'

mnc:

$ref: '#/components/schemas/Mnc'

PlmnIdList:

type: array

items:

$ref: '#/components/schemas/PlmnId'

PlmnInfo:

type: object

properties:

plmnId":

$ref: '#/components/schemas/PlmnId'

snssai:

$ref: '#/components/schemas/Snssai'

PlmnInfoList:

type: array

items:

$ref: '#/components/schemas/PlmnInfo'

GGnbId:

type: string

pattern: '^[0-9]{3}[0-9]{2,3}-(22|23|24|25|26|27|28|29|30|31|32)-[0-9]{1,10}'

GEnbId:

type: string

pattern: '^[0-9]{3}[0-9]{2,3}-(18|20|21|22)-[0-9]{1,7}'

GGnbIdList:

type: array

items:

$ref: '#/components/schemas/GGnbId'

GEnbIdList:

type: array

items:

$ref: '#/components/schemas/GEnbId'

NrPci:

type: integer

maximum: 503

NrTac:

type: integer

maximum: 16777215

Tai:

type: object

properties:

plmnId:

$ref: '#/components/schemas/PlmnId'

nrTac:

$ref: '#/components/schemas/NrTac'

BackhaulAddress:

type: object

properties:

gnbId:

$ref: '#/components/schemas/GnbId'

tai:

$ref: "#/components/schemas/Tai"

MappingSetIDBackhaulAddress:

type: object

properties:

setID:

type: integer

backhaulAddress:

$ref: '#/components/schemas/BackhaulAddress'

IntraRatEsActivationOriginalCellLoadParameters:

type: object

properties:

loadThreshold:

type: integer

timeDuration:

type: integer

IntraRatEsActivationCandidateCellsLoadParameters:

type: object

properties:

loadThreshold:

type: integer

timeDuration:

type: integer

IntraRatEsDeactivationCandidateCellsLoadParameters:

type: object

properties:

loadThreshold:

type: integer

timeDuration:

type: integer

EsNotAllowedTimePeriod:

type: object

properties:

startTimeandendTime:

type: string

periodOfDay:

type: string

daysOfWeekList:

type: string

listoftimeperiods:

type: string

InterRatEsActivationOriginalCellParameters:

type: object

properties:

loadThreshold:

type: integer

timeDuration:

type: integer

InterRatEsActivationCandidateCellParameters:

type: object

properties:

loadThreshold:

type: integer

timeDuration:

type: integer

InterRatEsDeactivationCandidateCellParameters:

type: object

properties:

loadThreshold:

type: integer

timeDuration:

type: integer

UeAccProbilityDist:

type: object

properties:

targetProbability:

type: integer

numberofpreamblessent:

type: integer

UeAccDelayProbilityDist:

type: object

properties:

targetProbability:

type: integer

accessdelay:

type: integer

NRPciList:

type: object

properties:

NRPci:

type: integer

CSonPciList:

type: object

properties:

NRPci:

type: integer

MaximumDeviationHoTrigger:

type: integer

minimum: -20

maximum: 20

MinimumTimeBetweenHoTriggerChange:

type: integer

minimum: 0

maximum: 604800

TstoreUEcntxt:

type: integer

minimum: 0

maximum: 1023

CellState:

type: string

enum:

- IDLE

- INACTIVE

- ACTIVE

CyclicPrefix:

type: string

enum:

- '15'

- '30'

- '60'

- '120'

TxDirection:

type: string

enum:

- DL

- UL

- DL and UL

BwpContext:

type: string

enum:

- DL

- UL

- SUL

IsInitialBwp:

type: string

enum:

- INITIAL

- OTHER

- SUL

QuotaType:

type: string

enum:

- STRICT

- FLOAT

IsESCoveredBy:

type: string

enum:

- NO

- PARTIAL

- FULL

RrmPolicyMember:

type: object

properties:

plmnId:

$ref: '#/components/schemas/PlmnId'

snssai:

$ref: '#/components/schemas/Snssai'

RrmPolicyMemberList:

type: array

items:

$ref: '#/components/schemas/RrmPolicyMember'

AddressWithVlan:

type: object

properties:

ipv4Address:

$ref: 'genericNrm.yaml#/components/schemas/Ipv4Addr'

ipv6Address:

$ref: 'genericNrm.yaml#/components/schemas/Ipv6Addr'

vlanId:

type: integer

minimum: 0

maximum: 4096

LocalAddress:

type: object

properties:

addressWithVlan:

$ref: '#/components/schemas/AddressWithVlan'

port:

type: integer

minimum: 0

maximum: 65535

RemoteAddress:

type: object

properties:

ipv4Address:

$ref: 'genericNrm.yaml#/components/schemas/Ipv4Addr'

ipv6Address:

$ref: 'genericNrm.yaml#/components/schemas/Ipv6Addr'

CellIndividualOffset:

type: object

properties:

rsrpOffsetSSB:

type: integer

rsrqOffsetSSB:

type: integer

sinrOffsetSSB:

type: integer

rsrpOffsetCSI-RS:

type: integer

rsrqOffsetCSI-RS:

type: integer

sinrOffsetCSI-RS:

type: integer

QOffsetRange:

type: integer

enum:

- -24

- -22

- -20

- -18

- -16

- -14

- -12

- -10

- -8

- -6

- -5

- -4

- -3

- -2

- -1

- 0

- 24

- 22

- 20

- 18

- 16

- 14

- 12

- 10

- 8

- 6

- 5

- 4

- 3

- 2

- 1

QOffsetRangeList:

type: object

properties:

rsrpOffsetSSB:

$ref: '#/components/schemas/QOffsetRange'

rsrqOffsetSSB:

$ref: '#/components/schemas/QOffsetRange'

sinrOffsetSSB:

$ref: '#/components/schemas/QOffsetRange'

rsrpOffsetCSI-RS:

$ref: '#/components/schemas/QOffsetRange'

rsrqOffsetCSI-RS:

$ref: '#/components/schemas/QOffsetRange'

sinrOffsetCSI-RS:

$ref: '#/components/schemas/QOffsetRange'

QOffsetFreq:

type: number

TReselectionNRSf:

type: integer

enum:

- 25

- 50

- 75

- 100

SsbPeriodicity:

type: integer

enum:

- 5

- 10

- 20

- 40

- 80

- 160

SsbDuration:

type: integer

enum:

- 1

- 2

- 3

- 4

- 5

SsbSubCarrierSpacing:

type: integer

enum:

- 15

- 30

- 120

- 240

CoverageShape:

type: integer

maximum: 65535

DigitalTilt:

type: integer

minimum: -900

maximum: 900

DigitalAzimuth:

type: integer

minimum: -1800

maximum: 1800

RSSetId:

type: integer

maximum: 4194303

RSSetType:

type: string

enum:

- RS1

- RS2

FrequencyDomainPara:

type: object

properties:

rimRSSubcarrierSpacing:

type: integer

rIMRSBandwidth:

type: integer

nrofGlobalRIMRSFrequencyCandidates:

type: integer

rimRSCommonCarrierReferencePoint:

type: integer

rimRSStartingFrequencyOffsetIdList:

type: array

items:

type: integer

SequenceDomainPara:

type: object

properties:

nrofRIMRSSequenceCandidatesofRS1:

type: integer

rimRSScrambleIdListofRS1:

type: array

items:

type: integer

nrofRIMRSSequenceCandidatesofRS2:

type: integer

rimRSScrambleIdListofRS2:

type: array

items:

type: integer

enableEnoughNotEnoughIndication:

type: string

enum:

- ENABLE

- DISABLE

RIMRSScrambleTimerMultiplier:

type: integer

RIMRSScrambleTimerOffset:

type: integer

TimeDomainPara:

type: object

properties:

dlULSwitchingPeriod1:

type: string

enum:

- MS0P5

- MS0P625

- MS1

- MS1P25

- MS2

- MS2P5

- MS3

- MS4

- MS5

- MS10

- MS20

symbolOffsetOfReferencePoint1:

type: integer

dlULSwitchingPeriod2:

type: string

enum:

- MS0P5

- MS0P625

- MS1

- MS1P25

- MS2

- MS2P5

- MS3

- MS4

- MS5

- MS10

- MS20

symbolOffsetOfReferencePoint2:

type: integer

totalnrofSetIdofRS1:

type: integer

totalnrofSetIdofRS2:

type: integer

nrofConsecutiveRIMRS1:

type: integer

nrofConsecutiveRIMRS2:

type: integer

consecutiveRIMRS1List:

type: array

items:

type: integer

consecutiveRIMRS2List:

type: array

items:

type: integer

enablenearfarIndicationRS1:

type: string

enum:

- ENABLE

- DISABLE

enablenearfarIndicationRS2:

type: string

enum:

- ENABLE

- DISABLE

RimRSReportInfo:

type: object

properties:

detectedSetID:

type: integer

propagationDelay:

type: integer

functionalityOfRIMRS:

type: string

enum:

- RS1

- RS2

- RS1forEnoughMitigation

- RS1forNotEnoughMitigation

RimRSReportConf:

type: object

properties:

reportIndicator:

type: string

enum:

- ENABLE

- DISABLE

reportInterval:

type: integer

nrofRIMRSReportInfo:

type: integer

maxPropagationDelay:

type: integer

rimRSReportInfoList:

type: array

items:

$ref: '#/components/schemas/RimRSReportInfo'

TceMappingInfo:

type: object

properties:

TceIPAddress:

oneOf:

- $ref: 'genericNrm.yaml#/components/schemas/Ipv4Addr'

- $ref: 'genericNrm.yaml#/components/schemas/Ipv6Addr'

TceID:

type: integer

PlmnTarget:

$ref: '#/components/schemas/PlmnId'

TceMappingInfoList:

type: array

items:

$ref: '#/components/schemas/TceMappingInfo'

#-------- Definition of abstract IOCs --------------------------------------------

RrmPolicy_-Attr:

type: object

properties:

resourceType:

type: string

rRMPolicyMemberList:

$ref: '#/components/schemas/RrmPolicyMemberList'

#-------- Definition of concrete IOCs --------------------------------------------

SubNetwork-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

$ref: 'genericNrm.yaml#/components/schemas/SubNetwork-Attr'

- $ref: 'genericNrm.yaml#/components/schemas/SubNetwork-ncO'

- type: object

properties:

SubNetwork:

$ref: '#/components/schemas/SubNetwork-Multiple'

ManagedElement:

$ref: '#/components/schemas/ManagedElement-Multiple'

NRFrequency:

$ref: '#/components/schemas/NRFrequency-Multiple'

ExternalGnbCuCpFunction:

$ref: '#/components/schemas/ExternalGnbCuCpFunction-Multiple'

ExternalENBFunction:

$ref: '#/components/schemas/ExternalENBFunction-Multiple'

EUtranFrequency:

$ref: '#/components/schemas/EUtranFrequency-Multiple'

DESManagementFunction:

$ref: '#/components/schemas/DESManagementFunction-Single'

DRACHOptimizationFunction:

$ref: '#/components/schemas/DRACHOptimizationFunction-Single'

DMROFunction:

$ref: '#/components/schemas/DMROFunction-Single'

DPCIConfigurationFunction:

$ref: '#/components/schemas/DPCIConfigurationFunction-Single'

CPCIConfigurationFunction:

$ref: '#/components/schemas/CPCIConfigurationFunction-Single'

CESManagementFunction:

$ref: '#/components/schemas/CESManagementFunction-Single'

Configurable5QISet:

$ref: '5gcNrm.yaml#/components/schemas/Configurable5QISet-Multiple'

RimRSGlobal:

$ref: '#/components/schemas/RimRSGlobal-Single'

Dynamic5QISet:

$ref: '5gcNrm.yaml#/components/schemas/Dynamic5QISet-Multiple'

ManagedElement-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

$ref: 'genericNrm.yaml#/components/schemas/ManagedElement-Attr'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedElement-ncO'

- type: object

properties:

GnbDuFunction:

$ref: '#/components/schemas/GnbDuFunction-Multiple'

GnbCuUpFunction:

$ref: '#/components/schemas/GnbCuUpFunction-Multiple'

GnbCuCpFunction:

$ref: '#/components/schemas/GnbCuCpFunction-Multiple'

DESManagementFunction:

$ref: '#/components/schemas/DESManagementFunction-Single'

DRACHOptimizationFunction:

$ref: '#/components/schemas/DRACHOptimizationFunction-Single'

DMROFunction:

$ref: '#/components/schemas/DMROFunction-Single'

DPCIConfigurationFunction:

$ref: '#/components/schemas/DPCIConfigurationFunction-Single'

CPCIConfigurationFunction:

$ref: '#/components/schemas/CPCIConfigurationFunction-Single'

CESManagementFunction:

$ref: '#/components/schemas/CESManagementFunction-Single'

Configurable5QISet:

$ref: '5gcNrm.yaml#/components/schemas/Configurable5QISet-Multiple'

Dynamic5QISet:

$ref: '5gcNrm.yaml#/components/schemas/Dynamic5QISet-Multiple'

GnbDuFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

gnbDuId:

$ref: '#/components/schemas/GnbDuId'

gnbDuName:

$ref: '#/components/schemas/GnbName'

gnbId:

$ref: '#/components/schemas/GnbId'

gnbIdLength:

$ref: '#/components/schemas/GnbIdLength'

rimRSReportConf:

$ref: '#/components/schemas/RimRSReportConf'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

RRMPolicyRatio:

$ref: '#/components/schemas/RRMPolicyRatio-Multiple'

NrCellDu:

$ref: '#/components/schemas/NrCellDu-Multiple'

Bwp-Multiple:

$ref: '#/components/schemas/Bwp-Multiple'

NrSectorCarrier-Multiple:

$ref: '#/components/schemas/NrSectorCarrier-Multiple'

EP_F1C:

$ref: '#/components/schemas/EP_F1C-Single'

EP_F1U:

$ref: '#/components/schemas/EP_F1U-Multiple'

DRACHOptimizationFunction:

$ref: '#/components/schemas/DRACHOptimizationFunction-Single'

GnbCuUpFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

gnbId:

$ref: '#/components/schemas/GnbId'

gnbIdLength:

$ref: '#/components/schemas/GnbIdLength'

gnbCuUpId:

$ref: '#/components/schemas/GnbCuUpId'

plmnInfoList:

$ref: '#/components/schemas/PlmnInfoList'

configurable5QISetRef:

$ref: 'genericNrm.yaml#/components/schemas/Dn'

dynamic5QISetRef:

$ref: 'genericNrm.yaml#/components/schemas/Dn'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

RRMPolicyRatio:

$ref: '#/components/schemas/RRMPolicyRatio-Multiple'

EP_E1:

$ref: '#/components/schemas/EP_E1-Single'

EP_XnU:

$ref: '#/components/schemas/EP_XnU-Multiple'

EP_F1U:

$ref: '#/components/schemas/EP_F1U-Multiple'

EP_NgU:

$ref: '#/components/schemas/EP_NgU-Multiple'

EP_X2U:

$ref: '#/components/schemas/EP_X2U-Multiple'

EP_S1U:

$ref: '#/components/schemas/EP_S1U-Multiple'

GnbCuCpFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

gnbId:

$ref: '#/components/schemas/GnbId'

gnbIdLength:

$ref: '#/components/schemas/GnbIdLength'

gnbCuName:

$ref: '#/components/schemas/GnbName'

plmnId:

$ref: '#/components/schemas/PlmnId'

x2BlackList:

$ref: '#/components/schemas/GGnbIdList'

xnBlackList:

$ref: '#/components/schemas/GGnbIdList'

x2WhiteList:

$ref: '#/components/schemas/GGnbIdList'

xnWhiteList:

$ref: '#/components/schemas/GGnbIdList'

x2XnHOBlackList:

$ref: '#/components/schemas/GEnbIdList'

mappingSetIDBackhaulAddress:

$ref: '#/components/schemas/MappingSetIDBackhaulAddress'

tceMappingInfoList:

$ref: '#/components/schemas/TceMappingInfoList'

configurable5QISetRef:

$ref: 'genericNrm.yaml#/components/schemas/Dn'

dynamic5QISetRef:

$ref: 'genericNrm.yaml#/components/schemas/Dn'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

RRMPolicyRatio:

$ref: '#/components/schemas/RRMPolicyRatio-Multiple'

NrCellCu:

$ref: '#/components/schemas/NrCellCu-Multiple'

EP_XnC:

$ref: '#/components/schemas/EP_XnC-Multiple'

EP_E1:

$ref: '#/components/schemas/EP_E1-Multiple'

EP_F1C:

$ref: '#/components/schemas/EP_F1C-Multiple'

EP_NgC:

$ref: '#/components/schemas/EP_NgC-Multiple'

EP_X2C:

$ref: '#/components/schemas/EP_X2C-Multiple'

DANRManagementFunction:

$ref: '#/components/schemas/DANRManagementFunction-Single'

DESManagementFunction:

$ref: '#/components/schemas/DESManagementFunction-Single'

DMROFunction:

$ref: '#/components/schemas/DMROFunction-Single'

NrCellCu-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

cellLocalId:

type: integer

plmnInfoList:

$ref: '#/components/schemas/PlmnInfoList'

nRFrequencyRef:

$ref: 'genericNrm.yaml#/components/schemas/Dn'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

RRMPolicyRatio:

$ref: '#/components/schemas/RRMPolicyRatio-Multiple'

NRCellRelation:

$ref: '#/components/schemas/NRCellRelation-Multiple'

EUtranCellRelation:

$ref: '#/components/schemas/EUtranCellRelation-Multiple'

NRFreqRelation:

$ref: '#/components/schemas/NRFreqRelation-Multiple'

EUtranFreqRelation:

$ref: '#/components/schemas/EUtranFreqRelation-Multiple'

DESManagementFunction:

$ref: '#/components/schemas/DESManagementFunction-Single'

DMROFunction:

$ref: '#/components/schemas/DMROFunction-Single'

CESManagementFunction:

$ref: '#/components/schemas/CESManagementFunction-Single'

DPCIConfigurationFunction:

$ref: '#/components/schemas/DPCIConfigurationFunction-Single'

NrCellDu-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

administrativeState:

$ref: 'genericNrm.yaml#/components/schemas/AdministrativeState'

operationalState:

$ref: 'genericNrm.yaml#/components/schemas/OperationalState'

cellLocalId:

type: integer

cellState:

$ref: '#/components/schemas/CellState'

plmnInfoList:

$ref: '#/components/schemas/PlmnInfoList'

nrPci:

$ref: '#/components/schemas/NrPci'

nrTac:

$ref: '#/components/schemas/NrTac'

arfcnDL:

type: integer

arfcnUL:

type: integer

arfcnSUL:

type: integer

bSChannelBwDL:

type: integer

bSChannelBwUL:

type: integer

bSChannelBwSUL:

type: integer

ssbFrequency:

type: integer

minimum: 0

maximum: 3279165

ssbPeriodicity:

$ref: '#/components/schemas/SsbPeriodicity'

ssbSubCarrierSpacing:

$ref: '#/components/schemas/SsbSubCarrierSpacing'

ssbOffset:

type: integer

minimum: 0

maximum: 159

ssbDuration:

$ref: '#/components/schemas/SsbDuration'

nrSectorCarrierRef:

type: array

items:

$ref: 'genericNrm.yaml#/components/schemas/Dn'

bwpRef:

type: array

items:

$ref: 'genericNrm.yaml#/components/schemas/Dn'

nRFrequencyRef:

$ref: 'genericNrm.yaml#/components/schemas/Dn'

victimSetRef:

$ref: 'genericNrm.yaml#/components/schemas/Dn'

aggressorSetRef:

$ref: 'genericNrm.yaml#/components/schemas/Dn'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

RRMPolicyRatio:

$ref: '#/components/schemas/RRMPolicyRatio-Multiple'

CPCIConfigurationFunction:

$ref: '#/components/schemas/CPCIConfigurationFunction-Single'

DRACHOptimizationFunction:

$ref: '#/components/schemas/DRACHOptimizationFunction-Single'

NRFrequency-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

type: object

properties:

absoluteFrequencySSB:

type: integer

minimum: 0

maximum: 3279165

ssbSubCarrierSpacing:

$ref: '#/components/schemas/SsbSubCarrierSpacing'

multiFrequencyBandListNR:

type: integer

minimum: 1

maximum: 256

EUtranFrequency-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

type: object

properties:

earfcnDL:

type: integer

minimum: 0

maximum: 262143

multiBandInfoListEutra:

type: integer

minimum: 1

maximum: 256

NrSectorCarrier-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

txDirection:

$ref: '#/components/schemas/TxDirection'

configuredMaxTxPower:

type: integer

arfcnDL:

type: integer

arfcnUL:

type: integer

bSChannelBwDL:

type: integer

bSChannelBwUL:

type: integer

sectorEquipmentFunctionRef:

$ref: 'genericNrm.yaml#/components/schemas/Dn'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

CommonBeamformingFunction:

$ref: '#/components/schemas/CommonBeamformingFunction-Single'

Bwp-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

bwpContext:

$ref: '#/components/schemas/BwpContext'

isInitialBwp:

$ref: '#/components/schemas/IsInitialBwp'

subCarrierSpacing:

type: integer

cyclicPrefix:

$ref: '#/components/schemas/CyclicPrefix'

startRB:

type: integer

numberOfRBs:

type: integer

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

CommonBeamformingFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

coverageShape:

$ref: '#/components/schemas/CoverageShape'

digitalAzimuth:

$ref: '#/components/schemas/DigitalAzimuth'

digitalTilt:

$ref: '#/components/schemas/DigitalTilt'

- type: object

properties:

Beam:

$ref: '#/components/schemas/Beam-Multiple'

Beam-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

beamIndex:

type: integer

beamType:

type: string

enum:

- SSB-BEAM

beamAzimuth:

type: integer

minimum: -1800

maximum: 1800

beamTilt:

type: integer

minimum: -900

maximum: 900

beamHorizWidth:

type: integer

minimum: 0

maximum: 3599

beamVertWidth:

type: integer

minimum: 0

maximum: 1800

RRMPolicyRatio-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: '#/components/schemas/RrmPolicy_-Attr'

- type: object

properties:

rRMPolicyMaxRatio:

type: integer

rRMPolicyMinRatio:

type: integer

rRMPolicyDedicatedRatio:

type: integer

NRCellRelation-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

type: object

properties:

nRTCI:

type: integer

cellIndividualOffset:

$ref: '#/components/schemas/CellIndividualOffset'

adjacentNRCellRef:

$ref: 'genericNrm.yaml#/components/schemas/Dn'

nRFrequencyRef:

$ref: 'genericNrm.yaml#/components/schemas/Dn'

isRemoveAllowed:

type: boolean

isHOAllowed:

type: boolean

isESCoveredBy:

$ref: '#/components/schemas/IsESCoveredBy'

isENDCAllowed:

type: boolean

isMLBAllowed:

type: boolean

EUtranCellRelation-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

adjacentEUtranCellRef:

$ref: 'genericNrm.yaml#/components/schemas/Dn'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

NRFreqRelation-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

type: object

properties:

offsetMO:

$ref: '#/components/schemas/QOffsetRangeList'

blackListEntry:

type: array

items:

type: integer

minimum: 0

maximum: 1007

blackListEntryIdleMode:

type: integer

cellReselectionPriority:

type: integer

cellReselectionSubPriority:

type: number

minimum: 0.2

maximum: 0.8

multipleOf: 0.2

pMax:

type: integer

minimum: -30

maximum: 33

qOffsetFreq:

$ref: '#/components/schemas/QOffsetFreq'

qQualMin:

type: number

qRxLevMin:

type: integer

minimum: -140

maximum: -44

threshXHighP:

type: integer

minimum: 0

maximum: 62

threshXHighQ:

type: integer

minimum: 0

maximum: 31

threshXLowP:

type: integer

minimum: 0

maximum: 62

threshXLowQ:

type: integer

minimum: 0

maximum: 31

tReselectionNr:

type: integer

minimum: 0

maximum: 7

tReselectionNRSfHigh:

$ref: '#/components/schemas/TReselectionNRSf'

tReselectionNRSfMedium:

$ref: '#/components/schemas/TReselectionNRSf'

nRFrequencyRef:

$ref: 'genericNrm.yaml#/components/schemas/Dn'

EUtranFreqRelation-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

type: object

properties:

cellIndividualOffset:

$ref: '#/components/schemas/CellIndividualOffset'

blackListEntry:

type: array

items:

type: integer

minimum: 0

maximum: 1007

blackListEntryIdleMode:

type: integer

cellReselectionPriority:

type: integer

cellReselectionSubPriority:

type: number

minimum: 0.2

maximum: 0.8

multipleOf: 0.2

pMax:

type: integer

minimum: -30

maximum: 33

qOffsetFreq:

$ref: '#/components/schemas/QOffsetFreq'

qQualMin:

type: number

qRxLevMin:

type: integer

minimum: -140

maximum: -44

threshXHighP:

type: integer

minimum: 0

maximum: 62

threshXHighQ:

type: integer

minimum: 0

maximum: 31

threshXLowP:

type: integer

minimum: 0

maximum: 62

threshXLowQ:

type: integer

minimum: 0

maximum: 31

tReselectionEutran:

type: integer

minimum: 0

maximum: 7

tReselectionNRSfHigh:

$ref: '#/components/schemas/TReselectionNRSf'

tReselectionNRSfMedium:

$ref: '#/components/schemas/TReselectionNRSf'

eUTranFrequencyRef:

$ref: 'genericNrm.yaml#/components/schemas/Dn'

DANRManagementFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

type: object

properties:

intrasystemANRManagementSwitch:

type: boolean

intersystemANRManagementSwitch:

type: boolean

DESManagementFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

type: object

properties:

desSwitch:

type: boolean

intraRatEsActivationOriginalCellLoadParameters:

$ref: "#/components/schemas/IntraRatEsActivationOriginalCellLoadParameters"

intraRatEsActivationCandidateCellsLoadParameters:

$ref: "#/components/schemas/IntraRatEsActivationCandidateCellsLoadParameters"

intraRatEsDeactivationCandidateCellsLoadParameters:

$ref: "#/components/schemas/IntraRatEsDeactivationCandidateCellsLoadParameters"

esNotAllowedTimePeriod:

$ref: "#/components/schemas/EsNotAllowedTimePeriod"

interRatEsActivationOriginalCellParameters:

$ref: "#/components/schemas/IntraRatEsActivationOriginalCellLoadParameters"

interRatEsActivationCandidateCellParameters:

$ref: "#/components/schemas/IntraRatEsActivationOriginalCellLoadParameters"

interRatEsDeactivationCandidateCellParameters:

$ref: "#/components/schemas/IntraRatEsActivationOriginalCellLoadParameters"

isProbingCapable:

type: string

enum:

- yes

- no

energySavingState:

type: string

enum:

- isNotEnergySaving

- isEnergySaving

DRACHOptimizationFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

type: object

properties:

drachOptimizationControl:

type: boolean

ueAccProbilityDist:

$ref: "#/components/schemas/UeAccProbilityDist"

ueAccDelayProbilityDist:

$ref: "#/components/schemas/UeAccDelayProbilityDist"

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

DMROFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

type: object

properties:

dmroControl:

type: boolean

maximumDeviationHoTrigger:

$ref: '#/components/schemas/MaximumDeviationHoTrigger'

minimumTimeBetweenHoTriggerChange:

$ref: '#/components/schemas/MinimumTimeBetweenHoTriggerChange'

tstoreUEcntxt:

$ref: '#/components/schemas/TstoreUEcntxt'

DPCIConfigurationFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

type: object

properties:

dPciConfigurationControl:

type: boolean

nRPciList:

$ref: "#/components/schemas/NRPciList"

CPCIConfigurationFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

type: object

properties:

cPciConfigurationControl:

type: boolean

cSonPciList:

$ref: "#/components/schemas/CSonPciList"

CESManagementFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

type: object

properties:

cesSwitch:

type: boolean

energySavingControl:

type: string

enum:

- toBeEnergySaving

- toBeNotEnergySaving

energySavingState:

type: string

enum:

- isNotEnergySaving

- isEnergySaving

RimRSGlobal-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

type: object

properties:

frequencyDomainPara:

$ref: '#/components/schemas/FrequencyDomainPara'

sequenceDomainPara:

$ref: '#/components/schemas/SequenceDomainPara'

timeDomainPara:

$ref: '#/components/schemas/TimeDomainPara'

RimRSSet:

$ref: '#/components/schemas/RimRSSet-Multiple'

RimRSSet-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

type: object

properties:

setId:

$ref: '#/components/schemas/RSSetId'

setType:

$ref: '#/components/schemas/RSSetType'

rimRSMonitoringStartTime:

type: string

rimRSMonitoringStopTime:

type: string

rimRSMonitoringWindowDuration:

type: integer

rimRSMonitoringWindowStartingOffset:

type: integer

rimRSMonitoringWindowPeriodicity:

type: integer

rimRSMonitoringOccasionInterval:

type: integer

rimRSMonitoringOccasionStartingOffset:

type: integer

nRCellDURefs:

$ref: 'genericNrm.yaml#/components/schemas/DnList'

ExternalGnbDuFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

gnbId:

$ref: '#/components/schemas/GnbId'

gnbIdLength:

$ref: '#/components/schemas/GnbIdLength'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

EP_F1C:

$ref: '#/components/schemas/EP_F1C-Multiple'

EP_F1U:

$ref: '#/components/schemas/EP_F1U-Multiple'

ExternalGnbCuUpFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

gnbId:

$ref: '#/components/schemas/GnbId'

gnbIdLength:

$ref: '#/components/schemas/GnbIdLength'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

EP_E1:

$ref: '#/components/schemas/EP_E1-Multiple'

EP_F1U:

$ref: '#/components/schemas/EP_F1U-Multiple'

EP_XnU:

$ref: '#/components/schemas/EP_XnU-Multiple'

ExternalGnbCuCpFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: >-

genericNrm.yaml#/components/schemas/ManagedFunction-Attr

- type: object

properties:

gnbId:

$ref: '#/components/schemas/GnbId'

gnbIdLength:

$ref: '#/components/schemas/GnbIdLength'

plmnId:

$ref: '#/components/schemas/PlmnId'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

ExternalNrCellCu:

$ref: '#/components/schemas/ExternalNrCellCu-Multiple'

EP_XnC:

$ref: '#/components/schemas/EP_XnC-Multiple'

EP_E1:

$ref: '#/components/schemas/EP_E1-Multiple'

EP_F1C:

$ref: '#/components/schemas/EP_F1C-Multiple'

ExternalNrCellCu-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

cellLocalId:

type: integer

nrPci:

$ref: '#/components/schemas/NrPci'

plmnIdList:

$ref: '#/components/schemas/PlmnIdList'

nRFrequencyRef:

$ref: 'genericNrm.yaml#/components/schemas/Dn'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

ExternalENBFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

eNBId:

type: integer

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

ExternalEUTranCell:

$ref: '#/components/schemas/ExternalEUTranCell-Multiple'

ExternalEUTranCell-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

EUtranFrequencyRef:

$ref: 'genericNrm.yaml#/components/schemas/Dn'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

EP_XnC-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: '#/components/schemas/LocalAddress'

remoteAddress:

$ref: '#/components/schemas/RemoteAddress'

EP_E1-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: '#/components/schemas/LocalAddress'

remoteAddress:

$ref: '#/components/schemas/RemoteAddress'

EP_F1C-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: '#/components/schemas/LocalAddress'

remoteAddress:

$ref: '#/components/schemas/RemoteAddress'

EP_NgC-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: '#/components/schemas/LocalAddress'

remoteAddress:

$ref: '#/components/schemas/RemoteAddress'

EP_X2C-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: '#/components/schemas/LocalAddress'

remoteAddress:

$ref: '#/components/schemas/RemoteAddress'

EP_XnU-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: '#/components/schemas/LocalAddress'

remoteAddress:

$ref: '#/components/schemas/RemoteAddress'

EP_F1U-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: '#/components/schemas/LocalAddress'

remoteAddress:

$ref: '#/components/schemas/RemoteAddress'

EP_NgU-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: '#/components/schemas/LocalAddress'

remoteAddress:

$ref: '#/components/schemas/RemoteAddress'

epTransportRefs:

$ref: 'genericNrm.yaml#/components/schemas/DnList'

EP_X2U-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: '#/components/schemas/LocalAddress'

remoteAddress:

$ref: '#/components/schemas/RemoteAddress'

EP_S1U-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: '#/components/schemas/LocalAddress'

remoteAddress:

$ref: '#/components/schemas/RemoteAddress'

#-------- Definition of JSON arrays for name-contained IOCs ----------------------

SubNetwork-Multiple:

type: array

items:

$ref: '#/components/schemas/SubNetwork-Single'

ManagedElement-Multiple:

type: array

items:

$ref: '#/components/schemas/ManagedElement-Single'

GnbDuFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/GnbDuFunction-Single'

GnbCuUpFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/GnbCuUpFunction-Single'

GnbCuCpFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/GnbCuCpFunction-Single'

NrCellDu-Multiple:

type: array

items:

$ref: '#/components/schemas/NrCellDu-Single'

NrCellCu-Multiple:

type: array

items:

$ref: '#/components/schemas/NrCellCu-Single'

NRFrequency-Multiple:

type: array

minItems: 1

items:

$ref: '#/components/schemas/NRFrequency-Single'

EUtranFrequency-Multiple:

type: array

minItems: 1

items:

$ref: '#/components/schemas/EUtranFrequency-Single'

NrSectorCarrier-Multiple:

type: array

items:

$ref: '#/components/schemas/NrSectorCarrier-Single'

Bwp-Multiple:

type: array

items:

$ref: '#/components/schemas/Bwp-Single'

Beam-Multiple:

type: array

items:

$ref: '#/components/schemas/Beam-Single'

RRMPolicyRatio-Multiple:

type: array

items:

$ref: '#/components/schemas/RRMPolicyRatio-Single'

NRCellRelation-Multiple:

type: array

items:

$ref: '#/components/schemas/NRCellRelation-Single'

EUtranCellRelation-Multiple:

type: array

items:

$ref: '#/components/schemas/EUtranCellRelation-Single'

NRFreqRelation-Multiple:

type: array

items:

$ref: '#/components/schemas/NRFreqRelation-Single'

EUtranFreqRelation-Multiple:

type: array

items:

$ref: '#/components/schemas/EUtranFreqRelation-Single'

RimRSSet-Multiple:

type: array

items:

$ref: '#/components/schemas/RimRSSet-Single'

ExternalGnbDuFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/ExternalGnbDuFunction-Single'

ExternalGnbCuUpFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/ExternalGnbCuUpFunction-Single'

ExternalGnbCuCpFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/ExternalGnbCuCpFunction-Single'

ExternalNrCellCu-Multiple:

type: array

items:

$ref: '#/components/schemas/ExternalNrCellCu-Single'

ExternalENBFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/ExternalENBFunction-Single'

ExternalEUTranCell-Multiple:

type: array

items:

$ref: '#/components/schemas/ExternalEUTranCell-Single'

EP_E1-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_E1-Single'

EP_XnC-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_XnC-Single'

EP_F1C-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_F1C-Single'

EP_NgC-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_NgC-Single'

EP_X2C-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_X2C-Single'

EP_XnU-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_XnU-Single'

EP_F1U-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_F1U-Single'

EP_NgU-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_NgU-Single'

EP_X2U-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_X2U-Single'

EP_S1U-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_S1U-Single'

#-------- Definitions in TS 28.541 for TS 28.532 ---------------------------------

resources-nrNrm:

oneOf:

- $ref: '#/components/schemas/SubNetwork-Single'

- $ref: '#/components/schemas/ManagedElement-Single'

- $ref: '#/components/schemas/GnbDuFunction-Single'

- $ref: '#/components/schemas/GnbCuUpFunction-Single'

- $ref: '#/components/schemas/GnbCuCpFunction-Single'

- $ref: '#/components/schemas/NrCellCu-Single'

- $ref: '#/components/schemas/NrCellDu-Single'

- $ref: '#/components/schemas/NRFrequency-Single'

- $ref: '#/components/schemas/EUtranFrequency-Single'

- $ref: '#/components/schemas/NrSectorCarrier-Single'

- $ref: '#/components/schemas/Bwp-Single'

- $ref: '#/components/schemas/CommonBeamformingFunction-Single'

- $ref: '#/components/schemas/Beam-Single'

- $ref: '#/components/schemas/RRMPolicyRatio-Single'

- $ref: '#/components/schemas/NRCellRelation-Single'

- $ref: '#/components/schemas/EUtranCellRelation-Single'

- $ref: '#/components/schemas/NRFreqRelation-Single'

- $ref: '#/components/schemas/EUtranFreqRelation-Single'

- $ref: '#/components/schemas/DANRManagementFunction-Single'

- $ref: '#/components/schemas/DESManagementFunction-Single'

- $ref: '#/components/schemas/DRACHOptimizationFunction-Single'

- $ref: '#/components/schemas/DMROFunction-Single'

- $ref: '#/components/schemas/DPCIConfigurationFunction-Single'

- $ref: '#/components/schemas/CPCIConfigurationFunction-Single'

- $ref: '#/components/schemas/CESManagementFunction-Single'

- $ref: '#/components/schemas/RimRSGlobal-Single'

- $ref: '#/components/schemas/RimRSSet-Single'

- $ref: '#/components/schemas/ExternalGnbDuFunction-Single'

- $ref: '#/components/schemas/ExternalGnbCuUpFunction-Single'

- $ref: '#/components/schemas/ExternalGnbCuCpFunction-Single'

- $ref: '#/components/schemas/ExternalNrCellCu-Single'

- $ref: '#/components/schemas/ExternalENBFunction-Single'

- $ref: '#/components/schemas/ExternalEUTranCell-Single'

- $ref: '#/components/schemas/EP_XnC-Single'

- $ref: '#/components/schemas/EP_E1-Single'

- $ref: '#/components/schemas/EP_F1C-Single'

- $ref: '#/components/schemas/EP_NgC-Single'

- $ref: '#/components/schemas/EP_X2C-Single'

- $ref: '#/components/schemas/EP_XnU-Single'

- $ref: '#/components/schemas/EP_F1U-Single'

- $ref: '#/components/schemas/EP_NgU-Single'

- $ref: '#/components/schemas/EP_X2U-Single'

- $ref: '#/components/schemas/EP_S1U-Single'
Annex E (normative):
YANG definitions for NR NRM

# E.1	General

This annex contains the YANG definitions for the NR and NG-RAN NRM, in accordance with NR and NG-RAN NRM information model definitions specified in clause 4.

# E.2	Void

# E.3	Void

# E.4	Void

# E.5	Modules

## E.5.1	module _3gpp-nr-nrm-beam@2019-11-22.yang

module _3gpp-nr-nrm-beam {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-nrnetwork-beam";

prefix "beam3gpp";

import _3gpp-nr-nrm-commonbeamformingfunction { prefix cbeamff3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-nr-nrm-gnbdufunction { prefix gnbdu3gpp; }

import _3gpp-nr-nrm-nrsectorcarrier { prefix nrsectcarr3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the Beam Information

Object Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2019-11-22 {

description "Initial revision";

reference "S5-197643";

}

typedef BeamType {

type enumeration {

enum SSB-BEAM;

}

}

grouping BeamGrp {

description "Represents the Beam IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

leaf beamIndex {

description "Index of the beam. ";

mandatory true;

type int32;

}

leaf beamType {

description "The type of the beam. ";

mandatory false;

type BeamType;

}

leaf beamAzimuth {

description "The azimuth of a beam transmission, which means the horizontal beamforming pointing angle (beam peak direction) in the (Phi) φ-axis in 1/10th degree resolution.  The pointing angle is the direction equal to the geometric centre of the half-power contour of the beam relative to the reference plane. Zero degree implies explicit antenna bearing (boresight). Positive angle implies clockwise from the antenna bearing.";

reference "3GPP TS 38.104, TS 38.901, TS 28.662";

mandatory false;

type int32 { range "-1800..1800"; }

units "0.1";

}

leaf beamTilt {

description "The tilt of a beam transmission, which means the vertical beamforming pointing angle (beam peak direction) in the (Theta) θ-axis in 1/10th degree resolution.

The pointing angle is the direction equal to the geometric centre of the half-power contour of the beam relative to the reference plane. Positive value implies downtilt.";

reference "3GPP TS 38.104, TS 38.901, TS 28.662";

mandatory false;

type int32 { range "-900..900"; }

units "0.1";

}

leaf beamHorizWidth {

description " The Horizontal beamWidth of a beam transmission, which means the horizontal beamforming half-power (3dB down) beamwidth in the (Phi) φ-axis in 1/10th degree resolution.";

reference "3GPP TS 38.104, TS 38.901";

mandatory false;

type int32 { range "0..3599"; }

units "0.1";

}

leaf beamVertWidth {

description " The Vertical beamWidth of a beam transmission, which means the vertical beamforming half-power (3dB down) beamwidth in the (Theta) θ-axis in 1/10th degree resolution.";

reference "3GPP TS 38.104, TS 38.901";

mandatory false;

type int32 { range "0..1800"; }

units "0.1";

}

}

augment "/me3gpp:ManagedElement/gnbdu3gpp:GNBDUFunction/nrsectcarr3gpp:NRSectorCarrier/cbeamff3gpp:CommonBeamformingFunction" {

list Beam {

description "Represents the per-Beam information required for, e.g. beam performance management utilizing measurements generated in the RAN. Can have spatial attributes of horizontal/azimuth (ie: Phi φ-axis) and vertical/tilt (ie: Theta θ-axis) beam pointing direction and beam width attributes.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses BeamGrp;

}

}

}

}

## E.5.1a	module _3gpp-nr-nrm-bwp.yang

module _3gpp-nr-nrm-bwp {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-bwp";

prefix "bwp3gpp";

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-gnbdufunction { prefix gnbdu3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the BWP Information Object Class

(IOC) that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2020-11-17 { reference CR-0410 ; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 { reference "Initial revision"; }

typedef CyclicPrefix {

type enumeration {

enum NORMAL;

enum EXTENDED;

}

}

typedef BwpContext {

type enumeration {

enum DL;

enum UL;

enum SUL;

}

}

typedef IsInitialBwp {

type enumeration {

enum INITIAL;

enum OTHER;

}

}

grouping BWPGrp {

description "Represents the BWP IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

leaf bwpContext {

description "Identifies whether the object is used for downlink, uplink

or supplementary uplink.";

mandatory true;

type BwpContext;

}

leaf isInitialBwp {

description "Identifies whether the object is used for initial or other

BWP.";

mandatory true;

type IsInitialBwp;

}

leaf subCarrierSpacing {

description "Subcarrier spacing configuration for a BWP.";

reference "3GPP TS 38.104";

mandatory true;

type uint32 { range "15 | 30 | 60 | 120"; }

units kHz;

}

leaf cyclicPrefix {

description "Cyclic prefix, which may be normal or extended.";

reference "3GPP TS 38.211";

mandatory true;

type CyclicPrefix;

}

leaf startRB {

description "Offset in common resource blocks to common resource block 0

for the applicable subcarrier spacing for a BWP.";

reference "N_BWP_start in 3GPP TS 38.211";

mandatory true;

type uint32;

}

leaf numberOfRBs {

description "Number of physical resource blocks for a BWP.";

reference "N_BWP_size in 3GPP TS 38.211";

mandatory true;

type uint32;

}

}

augment "/me3gpp:ManagedElement/gnbdu3gpp:GNBDUFunction" {

list BWP {

description "Represents a bandwidth part (BWP).";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses BWPGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## E.5.1b	module _3gpp-nr-nrm-commonbeamformingfunction@2019-11-22.yang

module _3gpp-nr-nrm-commonbeamformingfunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-nrnetwork-commonbeamformingfunction";

prefix "combeamformfunc3gpp";

import _3gpp-nr-nrm-nrsectorcarrier { prefix nrsectcarr3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-nr-nrm-gnbdufunction { prefix gnbdu3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the CommonBeamformingFuntion Information

Object Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2019-11-22 {

description "Initial revision";

reference "S5-197643";

}

grouping CommonBeamformingFunctionGrp {

description "Represents the CommonBeamformingFunction IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

leaf coverageShape {

description "Identifies the sector carrier coverage shape described by the envelope of the contained SSB beams. The coverage shape is implementation dependent.";

mandatory true;

type int32 { range "0..65535"; }

}

leaf digitalAzimuth {

description "Digitally-controlled azimuth through beamforming. It represents the horizontal pointing direction of the antenna relative to the antenna bore sight, representing the total non-mechanical horizontal pan of the selected coverageShape. Positive value gives azimuth to the right and negative value gives an azimuth to the left.";

reference "3GPP TS 38.104, TS 38.901, TS 28.662";

type int32 { range "-1800..1800"; }

units "0.1";

}

leaf digitalTilt {

description "Digitally-controlled tilt through beamforming. It represents the vertical pointing direction of the antenna relative to the antenna bore sight, representing the total non-mechanical vertical tilt of the selected coverageShape. Positive value gives downwards tilt and negative value gives upwards tilt.";

reference "3GPP TS 38.104, TS 38.901, TS 28.662";

type int32 { range "-900..900"; }

units "0.1";

}

}

augment "/me3gpp:ManagedElement/gnbdu3gpp:GNBDUFunction/nrsectcarr3gpp:NRSectorCarrier" {

list CommonBeamformingFunction {

description "Represents common beamforming functionality (eg: SSB beams) for the NRSectorCarrier.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses CommonBeamformingFunctionGrp;

}

}

}

}

## E.5.2	module_3gpp-nr-nrm-ep.yang

<CODE BEGINS>

module _3gpp-nr-nrm-ep {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-ep";

prefix "ep3gpp";

import _3gpp-common-ep-rp { prefix eprp3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-gnbcucpfunction { prefix gnbcucp3gpp; }

import _3gpp-nr-nrm-gnbcuupfunction { prefix gnbcuup3gpp; }

import _3gpp-nr-nrm-gnbdufunction { prefix gnbdu3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the NR related endpoint

Information Object Classes (IOCs) that are part of the NR Network

Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2021-03-03 { reference CR-0435 ; }

revision 2021-02-17 { reference CR-0470; }

revision 2020-11-17 { reference CR-0410; }

revision 2020-03-02 { reference S5-201191; }

revision 2019-06-17 { reference "Initial revision"; }

feature EPClassesUnderGNBCUCPFunction {

description "Endpoint classes shall be contained under GNBCUCPFunction";

}

feature EPClassesUnderGNBCUUPFunction {

description "Endpoint classes shall be contained under GNBCUUPFunction";

}

feature EPClassesUnderGNBDUFunction {

description "Endpoint classes shall be contained under GNBDUFunction";

}

grouping EP_E1Grp {

description "Represents the EP_E1 IOC.";

reference "3GPP TS 28.541, 3GPP TS 38.401";

uses eprp3gpp:EP_Common;

}

grouping EP_F1CGrp {

description "Represents the EP_F1C IOC.";

reference "3GPP TS 28.541, 3GPP TS 38.470";

uses eprp3gpp:EP_Common;

}

grouping EP_F1UGrp {

description "Represents the EP_F1U IOC.";

reference "3GPP TS 28.541, 3GPP TS 38.470";

uses eprp3gpp:EP_Common;

}

grouping EP_XnCGrp {

description "Represents the EP_XnC IOC.";

reference "3GPP TS 28.541, 3GPP TS 38.420";

uses eprp3gpp:EP_Common;

}

grouping EP_XnUGrp {

description "Represents the EP_XnU IOC.";

reference "3GPP TS 28.541, 3GPP TS 38.420";

uses eprp3gpp:EP_Common;

}

grouping EP_NgCGrp {

description "Represents the EP_NgC IOC.";

reference "3GPP TS 28.541, 3GPP TS 38.470";

uses eprp3gpp:EP_Common;

}

grouping EP_NgUGrp {

description "Represents the EP_NgU IOC.";

reference "3GPP TS 28.541, 3GPP TS 38.470";

uses eprp3gpp:EP_Common;

}

grouping EP_X2CGrp {

description "Represents the EP_X2C IOC.";

reference "3GPP TS 28.541, 3GPP TS 36.423";

uses eprp3gpp:EP_Common;

}

grouping EP_X2UGrp {

description "Represents the EP_X2U IOC.";

reference "3GPP TS 28.541, 3GPP TS 36.425";

uses eprp3gpp:EP_Common;

}

grouping EP_S1UGrp {

description "Represents the EP_S1U IOC.";

reference "3GPP TS 28.541, 3GPP TS 36.410";

uses eprp3gpp:EP_Common;

}

augment "/me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction" {

if-feature EPClassesUnderGNBCUCPFunction;

list EP_E1 {

description "Represents the local end point of the logical link,

supporting E1 interface between gNB-CU-CP and gNB-CU-UP.";

reference "3GPP TS 28.541, 3GPP TS 38.401";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_E1Grp;

}

}

list EP_F1C {

description "Represents the local end point of the control plane

interface (F1-C) between the DU and CU or CU-CP.";

reference "3GPP TS 28.541, 3GPP TS 38.470";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_F1CGrp;

}

}

list EP_NgC {

description "Represents the local end point of the control plane

interface (NG-C) between the gNB and NG-Core entity.";

reference "3GPP TS 28.541, 3GPP TS 38.470";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_NgCGrp;

}

}

list EP_XnC {

description "Represents the local gNB node end point of the logical

link, supporting Xn application protocols, to a neighbour NG-RAN node

(including gNB and ng-eNB). The Xn Application PDUs are carried over

SCTP/IP/Data link layer/Physical layer stack.";

reference "3GPP TS 28.541, 3GPP TS 38.420 subclause 7";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_XnCGrp;

}

}

list EP_X2C {

description "Represents the local end point of the logical link,

supporting X2-C application protocols used in EN-DC, to a neighbour

eNB or en-gNB node.";

reference "3GPP TS 28.541, 3GPP TS 36.423";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_X2CGrp;

}

}

}

augment "/me3gpp:ManagedElement/gnbcuup3gpp:GNBCUUPFunction" {

if-feature EPClassesUnderGNBCUUPFunction;

list EP_E1 {

description "Represents the local end point of the logical link,

supporting E1 interface between gNB-CU-CP and gNB-CU-UP.";

reference "3GPP TS 28.541, 3GPP TS 38.401";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_E1Grp;

}

}

list EP_F1U {

description "Represents the local end point of the user plane

interface (F1-U) between the DU and CU or CU-UP.";

reference "3GPP TS 28.541, 3GPP TS 38.470";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_F1UGrp;

}

}

list EP_NgU {

description "Represents the local end point of the NG user plane

(NG-U) interface between the gNB and UPF.";

reference "3GPP TS 28.541, 3GPP TS 38.470";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_NgUGrp;

}

}

list EP_XnU {

description "Represents the one end-point of a logical link supporting

the Xn user plane (Xn-U) interface. The Xn-U interface provides

non-guaranteed delivery of user plane PDUs between two NG-RAN nodes.";

reference "3GPP TS 28.541, 3GPP TS 38.420";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_XnUGrp;

}

}

list EP_X2U {

description "Represents the local end-point of a logical link supporting

the X2 user plane (X2-U) interface used in EN-DC.";

reference "3GPP TS 28.541, 3GPP TS 36.425";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_X2UGrp;

}

}

list EP_S1U {

description "Represents the local end point of the logical link,

supporting S1-U interface towards a S-GW node.";

reference "3GPP TS 28.541, 3GPP TS 36.410";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_S1UGrp;

}

}

}

augment "/me3gpp:ManagedElement/gnbdu3gpp:GNBDUFunction" {

if-feature EPClassesUnderGNBDUFunction;

list EP_F1C {

description "Represents the local end point of the control plane

interface (F1-C) between the DU and CU or CU-CP.";

reference "3GPP TS 28.541, 3GPP TS 38.470";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_F1CGrp;

}

}

list EP_F1U {

description "Represents the local end point of the user plane

interface (F1-U) between the DU and CU or CU-UP.";

reference "3GPP TS 28.541, 3GPP TS 38.470";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_F1UGrp;

}

}

}

}

<CODE ENDS>

## E.5.3	module _3gpp-nr-nrm-eutrancellrelation@2019-10-28.yang

module _3gpp-nr-nrm-eutrancellrelation {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-eutrancellrelation";

prefix "eutrancellrel3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-nr-nrm-gnbcucpfunction { prefix gnbcucp3gpp; }

import _3gpp-nr-nrm-nrcellcu { prefix nrcellcu3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the EUtranCellRelation Information

Object Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 {

description "Initial revision";

}

typedef ActionAllowed {

type enumeration {

enum YES;

enum NO;

}

}

typedef EnergySavingCoverage {

type enumeration {

enum YES;

enum NO;

enum PARTIAL;

}

}

grouping EUtranCellRelationGrp {

description "Represents the EUtranCellRelation IOC.";

reference "3GPP TS 28.541, EUtranRelation in 3GPP TS 28.658";

uses mf3gpp:ManagedFunctionGrp;

leaf tCI {

description "Target Cell Identifier. Consists of E-UTRAN Cell Global

Identifier (ECGI) and Physical Cell Identifier (PCI) of the target

cell. Identifies the target cell from the perspective of the parent

cell instance.";

mandatory true;

type uint64;

}

leaf isRemoveAllowed {

description "Indicates if the subject EUtranCellRelation can be removed

(deleted) or not. If YES, the subject EUtranCellRelation instance can

be removed (deleted). If NO, the subject EUtranCellRelation instance

shall not be removed (deleted) by any entity but an IRPManager.";

mandatory true;

type ActionAllowed;

}

leaf isHOAllowed {

description "Indicates if handover is allowed or prohibited. If YES,

handover is allowed from source cell to target cell. Source cell is

represented by the parent cell instance. Target cell is the adjacent

cell referenced by this EUtranCellRelation instance. If NO, handover

shall not be allowed.";

mandatory true;

type ActionAllowed;

}

leaf isENDCAllowed {

description "Indicates if EN-DC is allowed or prohibited. If TRUE,

the target cell is allowed to be used for EN-DC. The target cell is

referenced by the NRCellRelation that contains this isENDCAllowed.

If FALSE, EN-DC shall not be allowed.";

mandatory true;

type ActionAllowed;

}

leaf isICICInformationSendAllowed {

description "Indicates if ICIC (Inter Cell Interference Coordination)

load information message sending is allowed or prohibited. If YES,

ICIC load information message sending is allowed from source cell to

target cell. Source cell is represented by the parent cell instance.

Target cell is the adjacent cell referenced by this EUtranCellRelation

instance. If NO, ICIC load information message sending shall not be

allowed.";

reference "3GPP TS 36.423";

mandatory true;

type ActionAllowed;

}

leaf isLBAllowed {

description "Indicates if load balancing is allowed or prohibited from

source cell to target cell. If YES, load balancing is allowed from

source cell to target cell. Source cell is represented by the parent

cell instance. Target cell is the adjacent cell referenced by this

EUtranCellRelation instance. If NO, load balancing shall be prohibited

from source cell to target cell.";

mandatory true;

type ActionAllowed;

}

leaf isESCoveredBy {

description "Indicates whether the adjacent cell according to this

planning provides no, partial or full coverage for the parent cell

instance. Adjacent cells with this attribute equal to YES are

recommended to be considered as candidate cells to take over the

coverage when the original cell is about to be transferred to energy

saving state. The entirety of adjacent cells with this property equal

to PARTIAL are recommended to be considered as entirety of candidate

cells to take over the coverage when the original cell is about to be

transferred to energy saving state.";

mandatory true;

type EnergySavingCoverage;

}

leaf qOffset {

description "Offset applicable to a specific neighbouring cell used for

evaluating the cell as a candidate for cell re-selection. Corresponds

to parameter q-OffsetCell broadcast in SIB4 for intra-frequency cells

and in SIB5 for inter-frequency cells. Used for Mobility Robustness

Optimization.";

reference "3GPP TS 36.331";

mandatory true;

type types3gpp:QOffsetRange;

}

leaf cellIndividualOffset {

description "Offset applicable to a neighbouring cell. It is used for

evaluating the neighbouring cell for handover in connected mode. Used

by the HandOver parameter Optimization (HOO) function or Load

Balancing Optimization (LBO) function.";

reference "3GPP TS 36.331";

config false;

type types3gpp:QOffsetRange;

}

leaf adjacentCell {

description "Reference to an EUtranCellFDD/TDD or

ExternalEUtranCellFDD/TDD instance.";

mandatory true;

type types3gpp:DistinguishedName;

}

}

augment /me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction/nrcellcu3gpp:NRCellCU {

list EUtranCellRelation {

description "Represents a relation between an NR cell and an E-UTRAN cell.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EUtranCellRelationGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## E.5.4	module _3gpp-nr-nrm-eutranetwork@2019-06-17.yang

module _3gpp-nr-nrm-eutranetwork {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-eutranetwork";

prefix "eutranet3gpp";

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the EUtraNetwork Information Object

Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2019-06-17 {

description "Initial revision";

}

feature ExternalsUnderEUtraNetwork {

description "Classes representing external entities like EUtranFrequency,

ExternalENBFunction are contained under a EUtraNetwork list/class.";

}

grouping EUtraNetworkGrp {

description "Represents the EUtraNetwork IOC.";

reference "3GPP TS 28.541";

uses subnet3gpp:SubNetworkGrp;

}

list EUtraNetwork {

description "A subnetwork containing gNB external E-UTRAN entities.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EUtraNetworkGrp;

leaf-list parents {

description "Reference to all containg EUtraNetwork instances

in strict order from the root EUtraNetwork down to the immediate

parent EUtraNetwork.

If EUtraNetworks form a containment hierarchy this is

modeled using references between the child EUtraNetwork and the parent

EUtraNetworks.

This reference MUST NOT be present for the top level EUtraNetwork and

MUST be present for other EUtraNetworks.";

type leafref {

path "../../../EUtraNetwork/id";

}

}

leaf-list containedChildren{

description "Reference to all directly contained EUtraNetwork instances.

If EUtraNetworks form a containment hierarchy this is

modeled using references between the child EUtraNetwork and the parent

EUtraNetwork.";

type leafref {

path "../../../EUtraNetwork/id";

}

}

}

}

}

## E.5.5	module _3gpp-nr-nrm-eutranfreqrelation@2019-10-28.yang

module _3gpp-nr-nrm-eutranfreqrelation {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-eutranfreqrelation";

prefix "eutranfreqrel3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-gnbcucpfunction { prefix gnbcucp3gpp; }

import _3gpp-nr-nrm-nrcellcu { prefix nrcellcu3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the EUtranFreqRelation Information

Object Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 {

description "Initial revision";

}

grouping EUtranFreqRelationGrp {

description "Represents the EUtranFreqRelation IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

leaf cellIndividualOffset {

description "Offset applicable to a neighbouring cell. Used for

evaluating the neighbouring cell for handover in connected mode.

Used by the HandOver parameter Optimization (HOO) function or

Load Balancing Optimization (LBO) function.";

reference "cellIndividualOffset in MeasObjectEUTRA in 3GPP TS 38.331";

default 0;

type types3gpp:QOffsetRange;

}

leaf-list blackListEntry {

description "A list of Physical Cell Identities (PCIs) that are

blacklisted in E-UTRAN measurements.";

reference "3GPP TS 38.331";

min-elements 0;

type uint16 { range "0..1007"; }

}

leaf-list blackListEntryIdleMode {

description "A list of Physical Cell Identities (PCIs) that are

blacklisted in SIB4 and SIB5.";

min-elements 0;

type uint16 { range "0..1007"; }

}

leaf cellReselectionPriority {

description "The absolute priority of the carrier frequency used by the

cell reselection procedure. Value 0 means lowest priority. The value

must not already used by other RAT, i.e. equal priorities between RATs

are not supported. The UE behaviour when no value is entered is

specified in subclause 5.2.4.1 of 3GPP TS 38.304.";

reference "CellReselectionPriority in 3GPP TS 38.331, priority in

3GPP TS 38.304";

mandatory true;

type int32 { range "0..7"; }

}

leaf cellReselectionSubPriority {

description "Indicates a fractional value to be added to the value of

cellReselectionPriority to obtain the absolute priority of the

concerned carrier frequency for E-UTRA and NR.";

reference "3GPP TS 38.331";

type uint8 { range "2 | 4 | 6 | 8"; }

units "0.1";

}

leaf pMax {

description "Used for calculation of the parameter Pcompensation

(defined in 3GPP TS 38.304), at cell reselection to a cell.";

reference "PEMAX in 3GPP TS 38.101-1";

mandatory true;

type int32 { range "-30..33"; }

units dBm;

}

leaf qOffsetFreq {

description "The frequency specific offset applied when evaluating

candidates for cell reselection.";

type int32;

default 0;

}

leaf qQualMin {

description "Indicates the minimum required quality level in the cell.

Value 0 means that it is not sent and UE applies in such case the

(default) value of negative infinity for Qqualmin. Sent in SIB3 or

SIB5.";

reference "qQualMin in TS 38.304";

mandatory true;

type int32 { range "-34..-3 | 0"; }

units dB;

}

leaf qRxLevMin {

description "Indicates the required minimum received Reference Symbol

Received Power (RSRP) level in the (E-UTRA) frequency for cell

reselection. Broadcast in SIB3 or SIB5, depending on whether the

related frequency is intra- or inter-frequency. Resolution is 2.";

reference "Qrxlevmin in 3GPP TS 38.304";

mandatory true;

type int32 { range "-140..-44"; }

units dBm;

}

leaf threshXHighP {

description "Specifies the Srxlev threshold used by the UE when

reselecting towards a higher priority RAT/frequency than the current

serving frequency. Each frequency of NR and E-UTRAN might have a

specific threshold. Resolution is 2.";

reference "ThreshX, HighP in 3GPP TS 38.304";

mandatory true;

type int32 { range "0..62"; }

units dB;

}

leaf threshXHighQ {

description "Specifies the Squal threshold used by the UE when

reselecting towards a higher priority RAT/frequency than the current

serving frequency. Each frequency of NR and E-UTRAN might have a

specific threshold.";

reference "ThreshX, HighQ in 3GPP TS 38.304";

mandatory true;

type int32 { range 0..31; }

units dB;

}

leaf threshXLowP {

description "Specifies the Srxlev threshold used by the UE when

reselecting towards a lower priority RAT/frequency than the current

serving frequency. Each frequency of NR and E-UTRAN might have a

specific threshold. Resolution is 2.";

reference "ThreshX, LowP in 3GPP TS 38.304";

mandatory true;

type int32 { range "0..62"; }

units dB;

}

leaf threshXLowQ {

description "Specifies the Squal threshold used by the UE when

reselecting towards a lower priority RAT/frequency than the current

serving frequency. Each frequency of NR and E-UTRAN might have a

specific threshold.";

reference "ThreshX, LowQ in 3GPP TS 38.304";

mandatory false;

type int32 { range "0..31"; }

units dB;

}

leaf tReselectionEutra {

description "Cell reselection timer for intra frequency E-UTRA cell

reselection. May be used for Mobility Robustness Optimization.";

reference "t-ReselectionEUTRA in 3GPP TS 36.331 and in 3GPP TS 23.207";

mandatory true;

type uint8 { range "0..7"; }

units s;

}

leaf tReselectionEutraSfHigh {

description "The attribute tReselectionEutra (parameter TreselectionEUTRA

in 3GPP TS 38.304) multiplied with this scaling factor if the UE is in

high mobility state.";

reference "Speed dependent ScalingFactor for TreselectionEUTRA for high

mobility state in 3GPP TS 38.304";

mandatory true;

type uint8 { range "25 | 50 | 75 | 100"; }

units %;

}

leaf tReselectionEutraSfMedium {

description "The attribute tReselectionEutra (parameter TreselectionEUTRA

in 3GPP TS 38.304) multiplied with this scaling factor if the UE is in

medium mobility state.";

reference "Speed dependent ScalingFactor for TreselectionEUTRA for medium

mobility state in 3GPP TS 38.304";

mandatory true;

type uint8 { range "25 | 50 | 75 | 100"; }

units %;

}

leaf eUtranFrequencyRef {

description "Reference to a corresponding EUtranFrequency instance.";

mandatory true;

type types3gpp:DistinguishedName;

}

}

augment /me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction/nrcellcu3gpp:NRCellCU {

list EUtranFreqRelation {

description "Represents a frequency relation between an NR cell and an

E-UTRAN cell.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EUtranFreqRelationGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## E.5.6	module _3gpp-nr-nrm-eutranfrequency@2019-10-28.yang

module _3gpp-nr-nrm-eutranfrequency {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-eutranfrequency";

prefix "eutraneteutranfreq3gpp";

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-nr-nrm-eutranetwork { prefix eutranet3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the EUtranFrequency Information

Object Class (IOC), that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM),

3GPP TS 28.658 (E-UTRAN) Network Resource Model (NRM)";

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 {

description "Initial revision";

}

grouping EUtranFrequencyGrp {

description "Represents the EUtranFrequency IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

leaf earfcnDL {

description "Specifies the channel number for the central DL frequency.";

reference "3GPP TS 36.101";

mandatory true;

type uint32 { range "0..262143"; }

}

leaf-list multiBandInfoListEutra {

description "List of additional frequency bands the frequency belongs to.";

config false;

min-elements 0;

type uint16 { range "1..256"; }

}

}

grouping EUtranFrequencyWrapper {

list EUtranFrequency {

description "Represents certain E-UTRAN frequency properties.";

reference "3GPP TS 28.658";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EUtranFrequencyGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

augment "/subnet3gpp:SubNetwork" {

if-feature subnet3gpp:ExternalsUnderSubNetwork ;

uses EUtranFrequencyWrapper ;

}

augment "/eutranet3gpp:EUtraNetwork" {

if-feature eutranet3gpp:ExternalsUnderEUtraNetwork;

uses EUtranFrequencyWrapper ;

}

}

## E.5.7	module _3gpp-nr-nrm-externalamffunction@2019-10-28.yang

module _3gpp-nr-nrm-externalamffunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-externalamffunction";

prefix "extamf3gpp";

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-nr-nrm-nrnetwork { prefix nrnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the ExternalAMFFunction Information

Object Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 {

description "Initial revision";

}

grouping ExternalAMFFunctionGrp {

description "Represents the ExternalAMFFunction IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least

one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a

Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

container aMFIdentifier {

presence true;

description "An AMF identifier, comprising an AMF Region ID, an AMF Set ID and an AMF Pointer.";

uses types3gpp:AmfIdentifier;

}

}

grouping ExternalAMFFunctionWrapper {

list ExternalAMFFunction {

description "Represents the properties, known by the management

function, of a AMFFunction managed by another management

function.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalAMFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

augment "/subnet3gpp:SubNetwork" {

if-feature subnet3gpp:ExternalsUnderSubNetwork ;

uses ExternalAMFFunctionWrapper;

}

augment "/nrnet3gpp:NRNetwork" {

if-feature nrnet3gpp:ExternalsUnderNRNetwork;

uses ExternalAMFFunctionWrapper;

}

}

## E.5.8	module _3gpp-nr-nrm-externalenbfunction@2019-10-28.yang

module _3gpp-nr-nrm-externalenbfunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-externalenbfunction";

prefix "extenb3gpp";

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-nr-nrm-eutranetwork { prefix eutranet3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the ExternalENBFunction

Information Object Class (IOC) that is part of the NR Network Resource

Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM),

3GPP TS 28.658 (E-UTRAN) Network Resource Model (NRM)";

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 {

description "Initial revision";

}

grouping ExternalENBFunctionGrp {

description "Represets the ExternalENBFunction IOC.";

reference "3GPP TS 28.658";

uses mf3gpp:ManagedFunctionGrp;

leaf eNBId {

description "Unambiguously identifies an eNodeB within a PLMN.";

reference "3GPP TS 36.413, 3GPP TS 36.300";

mandatory true;

type int32 { range "0..268435455"; } // Representing 28 bit eNB ID.

// 18, 20 and 21 bit eNB IDs also

// allowed.

}

}

grouping ExternalENBFunctionWrapper {

list ExternalENBFunction {

description "Represents an external eNB functionality.";

reference "3GPP TS 28.658";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalENBFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

augment "/subnet3gpp:SubNetwork" {

if-feature subnet3gpp:ExternalsUnderSubNetwork ;

uses ExternalENBFunctionWrapper;

}

augment "/eutranet3gpp:EUtraNetwork" {

if-feature eutranet3gpp:ExternalsUnderEUtraNetwork;

uses ExternalENBFunctionWrapper;

}

}

## E.5.9	module_3gpp-nr-nrm-externaleutrancell@2019-10-28.yang

module _3gpp-nr-nrm-externaleutrancell {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-externaleutrancell";

prefix "exteutrancell3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-nr-nrm-eutranetwork { prefix eutranet3gpp; }

import _3gpp-nr-nrm-externalenbfunction { prefix extenb3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the ExternalEUtranCellFDD and

ExternalEUtranCellTDD Information Object Classes (IOCs) that are part

of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM),

3GPP TS 28.658 (E-UTRAN) Network Resource Model (NRM)";

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 {

description "Initial revision";

}

grouping ExternalEUtranGenericCellGrp {

description "Represents the ExternalEUtranGenericCell IOC.";

reference "3GPP TS 28.658";

uses mf3gpp:ManagedFunctionGrp;

leaf pci {

description "The Physical Cell Identity (PCI) of the cell (for

NM-Centralized, EM-Centralized and Distributed PCI assignment cases).

In the case of NM-Centralized PCI assignment, see 3GPP TS 36.300.";

reference "3GPP TS 36.211";

mandatory true;

type int32 { range "0..503"; }

}

list plmnIdList {

description "List of unique identities for PLMNs. A cell can broadcast

up to 6 PLMN IDs. This is to support the case that one cell can be

used by up to 6 operator's core networks. The PLMN(s) included in this

list will use the same single tracking area code (TAC) and the same

Cell Identity (cellLocalId) for sharing the radio access network

resources. One member of plmnIdList is the primary PLMN ID. A PLMN ID

included in this list cannot be included in the cellAccessInfoList.

The PLMN ID is composed of a Mobile Country Code (MCC) and a Mobile

Network Code (MNC).";

reference "3GPP TS 36.300, 3GPP TS 36.331, 3GPP TS 23.003";

key "mcc mnc";

min-elements 1;

max-elements 6;

uses types3gpp:PLMNId;

}

leaf cellLocalId {

description "Unambiguously identifies a cell within an eNodeB.";

reference "NCI defined in 3GPP TS 38.300";

type int32 {range "0..255"; }

}

leaf eNBId {

description "Unambiguously identifies an eNodeB within a PLMN.";

reference "3GPP TS 36.413, 3GPP TS 36.300";

mandatory true;

type int32 { range "0..268435455"; } // Representing 28 bit eNB ID.

// 18, 20 and 21 bit eNB IDs also

// allowed.

}

}

grouping ExternalEUtranCellFDDGrp {

description "Represents the ExternalEUtranCellFDD IOC.";

reference "3GPP TS 28.658";

uses ExternalEUtranGenericCellGrp;

leaf earfcnDL {

description "The channel number for the central DL frequency.";

reference "3GPP TS 36.101";

mandatory true;

type int32 { range "0..17999 | 46590..262143"; }

}

leaf earfcnUL {

description "The channel number for the central UL frequency. Value 0

means that the UL channel number is N/A for the DL-only bands.";

reference "3GPP TS 36.101";

mandatory true;

type int32 { range "0 | 18000..35999 | 46590..262143"; }

}

}

grouping ExternalEUtranCellTDDGrp {

description "Represents the ExternalEUtranCellTDD IOC.";

reference "3GPP TS 28.658";

uses ExternalEUtranGenericCellGrp;

leaf earfcn {

description "The frequency number for the central frequency.";

reference "3GPP TS 36.104";

mandatory true;

type int32 { range "36000..262143"; }

}

}

grouping ExternalEUtranCellFDDWrapper {

list ExternalEUtranCellFDD {

description "Represents the common properties of external E-UTRAN FDD

cell provided by eNB or NG-RAN FDD cell provided by ng-eNB.";

reference "3GPP TS 28.658";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalEUtranCellFDDGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

grouping ExternalEUtranCellTDDWrapper {

list ExternalEUtranCellTDD {

description "Represents the common properties of external E-UTRAN cell

TDD provided by eNB or NG-RAN TDD cell provided by ng-eNB.";

reference "3GPP TS 28.658";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalEUtranCellTDDGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

augment "/subnet3gpp:SubNetwork/extenb3gpp:ExternalENBFunction" {

if-feature subnet3gpp:ExternalsUnderSubNetwork;

uses ExternalEUtranCellFDDWrapper;

}

augment "/eutranet3gpp:EUtraNetwork/extenb3gpp:ExternalENBFunction" {

if-feature eutranet3gpp:ExternalsUnderEUtraNetwork;

uses ExternalEUtranCellFDDWrapper;

}

augment "/subnet3gpp:SubNetwork/extenb3gpp:ExternalENBFunction" {

if-feature subnet3gpp:ExternalsUnderSubNetwork;

uses ExternalEUtranCellTDDWrapper;

}

augment "/eutranet3gpp:EUtraNetwork/extenb3gpp:ExternalENBFunction" {

if-feature eutranet3gpp:ExternalsUnderEUtraNetwork;

uses ExternalEUtranCellTDDWrapper;

}

}

## E.5.10	module _3gpp-nr-nrm-externalgnbcucpfunction@2019-10-28.yang

module _3gpp-nr-nrm-externalgnbcucpfunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-externalgnbcucpfunction";

prefix "extgnbcucp3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-nr-nrm-nrnetwork { prefix nrnet3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the ExternalGNBCUCPFunction

Information Object Class (IOC), that is part of the NR Network Resource

Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 {

description "Initial revision";

}

grouping ExternalGNBCUCPFunctionGrp {

description "Represets the ExternalGNBCUCPFunction IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

leaf gNBId {

description "Identifies a gNB within a PLMN.";

reference "gNB Identifier (gNB ID) in 3GPP TS 38.300, Global gNB ID

in 3GPP TS 38.413";

mandatory true;

type int64 { range "0..4294967295"; }

}

leaf gNBIdLength {

description "Indicates the number of bits for encoding the gNB ID.";

reference "gNB ID in 3GPP TS 38.300, Global gNB ID in 3GPP TS 38.413";

mandatory true;

type int32 { range "22..32"; }

}

list pLMNId {

description "Specifies the PLMN identifier to be used as part of the

global RAN node identity.";

key "mcc mnc";

min-elements 1;

max-elements 1;

uses types3gpp:PLMNId;

}

}

grouping ExternalGNBCUCPFunctionWrapper {

list ExternalGNBCUCPFunction {

description "Represents the properties, known by the management function,

of a GNBCUCPFunction managed by another management function.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalGNBCUCPFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

augment "/subnet3gpp:SubNetwork" {

if-feature subnet3gpp:ExternalsUnderSubNetwork ;

uses ExternalGNBCUCPFunctionWrapper;

}

augment "/nrnet3gpp:NRNetwork" {

if-feature nrnet3gpp:ExternalsUnderNRNetwork;

uses ExternalGNBCUCPFunctionWrapper;

}

}

## E.5.11	module _3gpp-nr-nrm-externalgnbcuupfunction@2019-10-28.yang

module _3gpp-nr-nrm-externalgnbcuupfunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-externalgnbcuupfunction";

prefix "extgnbcuup3gpp";

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-nr-nrm-nrnetwork { prefix nrnet3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the ExternalGNBCUUPFunction

Information Object Class (IOC), that is part of the NR Network

Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 {

description "Initial revision";

}

grouping ExternalGNBCUUPFunctionGrp {

description "Represets the ExternalGNBCUUPFunction IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

leaf gNBId {

description "Identifies a gNB within a PLMN.";

reference "gNB Identifier (gNB ID) in 3GPP TS 38.300, Global gNB ID

in 3GPP TS 38.413";

mandatory true;

type int64 { range "0..4294967295"; }

}

leaf gNBIdLength {

description "Indicates the number of bits for encoding the gNB ID.";

reference "gNB ID in 3GPP TS 38.300, Global gNB ID in 3GPP TS 38.413";

mandatory true;

type int32 { range "22..32"; }

}

}

grouping ExternalGNBCUUPFunctionWrapper {

list ExternalGNBCUUPFunction {

description "Represents the properties, known by the management function,

of a GNBCUUPFunction managed by another management function.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalGNBCUUPFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

augment "/subnet3gpp:SubNetwork" {

if-feature subnet3gpp:ExternalsUnderSubNetwork ;

uses ExternalGNBCUUPFunctionWrapper;

}

augment "/nrnet3gpp:NRNetwork" {

if-feature nrnet3gpp:ExternalsUnderNRNetwork;

uses ExternalGNBCUUPFunctionWrapper;

}

}

## E.5.12	module _3gpp-nr-nrm-externalgnbdufunction@2019-10-28.yang

module _3gpp-nr-nrm-externalgnbdufunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-externalgnbdufunction";

prefix "extgnbdu3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-nr-nrm-nrnetwork { prefix nrnet3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the ExternalGNBDUFunction

Information Object Class (IOC) that is part of the NR Network Resource

Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 {

description "Initial revision";

}

grouping ExternalGNBDUFunctionGrp {

description "Represets the ExternalGNBDUFunction IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

leaf gNBId {

description "Identifies a gNB within a PLMN.";

reference "gNB Identifier (gNB ID) in 3GPP TS 38.300, Global gNB ID

in 3GPP TS 38.413";

mandatory true;

type int64 { range "0..4294967295"; }

}

leaf gNBIdLength {

description "Indicates the number of bits for encoding the gNB ID.";

reference "gNB ID in 3GPP TS 38.300, Global gNB ID in 3GPP TS 38.413";

mandatory true;

type int32 { range "22..32"; }

}

list pLMNId {

description "Specifies the PLMN identifier to be used as part of the

global RAN node identity.";

key "mcc mnc";

min-elements 1;

max-elements 1;

uses types3gpp:PLMNId;

}

}

grouping ExternalGNBDUFunctionWrapper {

list ExternalGNBDUFunction {

description "Represents the properties, known by the management function,

of a GNBDUFunction managed by another management function.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalGNBDUFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

augment "/subnet3gpp:SubNetwork" {

if-feature subnet3gpp:ExternalsUnderSubNetwork ;

uses ExternalGNBDUFunctionWrapper;

}

augment "/nrnet3gpp:NRNetwork" {

if-feature nrnet3gpp:ExternalsUnderNRNetwork;

uses ExternalGNBDUFunctionWrapper;

}

}

## E.5.13	module _3gpp-nr-nrm-externalnrcellcu@2019-10-28.yang

module _3gpp-nr-nrm-externalnrcellcu {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-externalnrcellcu";

prefix "extnrcellcu3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-nr-nrm-nrnetwork { prefix nrnet3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-nr-nrm-externalgnbcucpfunction { prefix extgnbcucp3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the ExternalNRCellCU Information

Object Class (IOC), that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 {

description "Initial revision";

}

grouping ExternalNRCellCUGrp {

description "Represents the ExternalNRCellCU IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

leaf cellLocalId {

description "Identifies an NR cell of a gNB. Together with corresponding

gNB ID it forms the NR Cell Identifier (NCI).";

reference "NCI in 3GPP TS 38.300";

mandatory true;

type int32 {range "0..16383"; }

}

leaf nRPCI {

description "The Physical Cell Identity (PCI) of the NR cell.";

reference "3GPP TS 36.211";

mandatory true;

type int32 { range "0..1007"; }

}

list pLMNIdList {

description "Defines which PLMNs that are assumed to be served by the

NR cell in another gNB CU-CP. This list is either updated by the

managed element itself (e.g. due to ANR, signalling over Xn, etc.) or

by consumer over the standard interface.";

key "mcc mnc";

min-elements 1;

max-elements 12;

uses types3gpp:PLMNId;

}

leaf nRFrequencyRef {

description "Reference to corresponding NRFrequency instance.";

mandatory true;

type types3gpp:DistinguishedName;

}

}

grouping ExternalNRCellCUWrapper {

list ExternalNRCellCU {

description "Represents the properties of an NRCellCU controlled by

another Management Service Provider.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalNRCellCUGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

augment "/subnet3gpp:SubNetwork/extgnbcucp3gpp:ExternalGNBCUCPFunction" {

if-feature subnet3gpp:ExternalsUnderSubNetwork ;

uses ExternalNRCellCUWrapper;

}

augment "/nrnet3gpp:NRNetwork/extgnbcucp3gpp:ExternalGNBCUCPFunction" {

if-feature nrnet3gpp:ExternalsUnderNRNetwork;

uses ExternalNRCellCUWrapper;

}

}

## E.5.14	module _3gpp-nr-nrm-externalservinggwfunction@2019-10-28.yang

module _3gpp-nr-nrm-externalservinggwfunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-externalservinggwfunction";

prefix "extservgw3gpp";

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-nr-nrm-eutranetwork { prefix eutranet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the ExternalServingGWFunction

Information Object Class (IOC) that is part of the NR Network Resource

Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 {

description "Initial revision";

}

grouping ExternalServingGWFunctionGrp {

description "Represents the ExternalServingGWFunction IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

}

grouping ExternalServingGWFunctionWrapper {

list ExternalServingGWFunction {

description "Represents the properties, known by the management

function, of a ServingGWFunction managed by another management

function.";

reference "3GPP TS 28.658";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalServingGWFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

augment "/subnet3gpp:SubNetwork" {

if-feature subnet3gpp:ExternalsUnderSubNetwork ;

uses ExternalServingGWFunctionWrapper;

}

augment "/eutranet3gpp:EUtraNetwork" {

if-feature eutranet3gpp:ExternalsUnderEUtraNetwork;

uses ExternalServingGWFunctionWrapper;

}

}

## E.5.15	module _3gpp-nr-nrm-externalupffunction@2019-10-28.yang

module _3gpp-nr-nrm-externalupffunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-externalupffunction";

prefix "extupf3gpp";

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-nr-nrm-nrnetwork { prefix nrnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the ExternalUPFFunction Information

Object Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 {

description "Initial revision";

}

grouping ExternalUPFFunctionGrp {

description "Represents the ExternalUPFFunction IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

}

grouping ExternalUPFFunctionWrapper {

list ExternalUPFFunction {

description "Represents the properties, known by the management

function, of a UPFFunction managed by another management

function.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalUPFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

augment "/subnet3gpp:SubNetwork" {

if-feature subnet3gpp:ExternalsUnderSubNetwork ;

uses ExternalUPFFunctionWrapper;

}

augment "/nrnet3gpp:NRNetwork" {

if-feature nrnet3gpp:ExternalsUnderNRNetwork;

uses ExternalUPFFunctionWrapper;

}

}

## E.5.16	module _3gpp-nr-nrm-gnbcucpfunction.yang

module _3gpp-nr-nrm-gnbcucpfunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-gnbcucpfunction";

prefix "gnbcucp3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-rrmpolicy { prefix nrrrmpolicy3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the GNBCUCPFunction Information

Object Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2020-10-02 { reference CR-0384 ; }

revision 2020-08-06 { reference "CR-0333"; }

revision 2020-08-03 { reference "CR-0321"; }

revision 2020-06-03 { reference "CR-0286"; }

revision 2020-05-08 { reference S5-203316 ; }

revision 2020-04-28 { reference "0260"; }

revision 2020-02-14 { reference S5-20XXXX ; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 {

description "Initial revision";

}

feature DESManagementFunction {

description "Classs representing Distributed SON or Domain-Centralized SON Energy Saving feature";

}

feature DANRManagementFunction {

description "Classs representing D-SON function of ANR Management feature";

}

feature DMROFunction {

description "Classs representing D-SON function of MRO feature";

}

grouping GNBCUCPFunctionGrp {

description "Represents the GNBCUCPFunction IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

uses nrrrmpolicy3gpp:RRMPolicy_Grp;

leaf gNBId {

description "Identifies a gNB within a PLMN. The gNB Identifier (gNB ID)

is part of the NR Cell Identifier (NCI) of the gNB cells.";

reference "gNB ID in 3GPP TS 38.300, Global gNB ID in 3GPP TS 38.413";

mandatory true;

type int64 { range "0..4294967295"; }

}

leaf gNBIdLength {

description "Indicates the number of bits for encoding the gNB ID.";

reference "gNB ID in 3GPP TS 38.300, Global gNB ID in 3GPP TS 38.413";

mandatory true;

type int32 { range "22..32"; }

}

leaf gNBCUName {

description "Identifies the Central Unit of an gNB.";

reference "3GPP TS 38.473";

mandatory true;

type string { length "1..150"; }

}

list pLMNId {

description "The PLMN identifier to be used as part of the global RAN

node identity.";

key "mcc mnc";

min-elements 1;

max-elements 1;

uses types3gpp:PLMNId;

}

leaf-list x2BlackList {

type string;

description "List of nodes to which X2 connections are prohibited.";

}

leaf-list x2WhiteList {

type string;

description "List of nodes to which X2 connections are enforced.";

}

leaf-list xnBlackList {

type string;

description "List of nodes to which Xn connections are prohibited.";

}

leaf-list xnWhiteList {

type string;

description "List of nodes to which X2 connections are enforced.";

}

leaf-list xnHOBlackList {

type string;

description "List of nodes to which handovers over  Xn are prohibited.";

}

leaf configurable5QISetRef {

type types3gpp:DistinguishedName;

description "DN of the Configurable5QISet that the GNBCUCPFunction supports (is associated to).";

}

leaf-list x2HOBlackList {

type string;

description "List of nodes to which handovers over X2 are prohibited.";

}

leaf dynamic5QISetRef {

type types3gpp:DistinguishedName;

description "DN of the Dynamic5QISet that the GNBCUCPFunction supports (is associated to).";

}

}

augment "/me3gpp:ManagedElement" {

list GNBCUCPFunction {

description "Represents the logical function CU-CP of gNB and en-gNB.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses GNBCUCPFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## E.5.17	module _3gpp-nr-nrm-gnbcuupfunction.yang

module _3gpp-nr-nrm-gnbcuupfunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-gnbcuupfunction";

prefix "gnbcuup3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-rrmpolicy { prefix nrrrmpolicy3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the GNBCUUPFunction Information

Object Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2020-11-05 { reference CR-0412 ; }

revision 2020-08-06 { reference "CR-0333"; }

revision 2020-08-03 { reference "CR-0321"; }

revision 2020-06-03 { reference "CR-0286"; }

revision 2020-05-28 { reference "CR-0318"; }

revision 2020-03-12 { reference "SP-200233 S5-201547"; }

revision 2020-02-14 { reference S5-20XXXX ; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-08-21 { reference "Initial revision"; }

grouping TAIGrp {

description "Tracking Area Identity";

list pLMNId {

key "mcc mnc";

description "PLMN IDs for the Tracking area";

uses types3gpp:PLMNId;

}

leaf nRTAC {

type int64;

description "Identity of the common Tracking Area Code for the PLMNs

allowedValues:

a) It is the TAC or Extended-TAC.

b) A cell can only broadcast one TAC or Extended-TAC.

See TS 36.300, subclause 10.1.7 (PLMNID and TAC relation).

c) TAC is defined in subclause 19.4.2.3 of 3GPP TS 23.003 and

Extended-TAC is defined in subclause 9.3.1.29 of 3GPP TS 38.473.

d) For a 5G SA (Stand Alone), it has a non-null value.";

}

}

grouping BackhaulAddressGrp {

description "Indicates the backhauladdress of gNB.";

leaf gNBId {

type uint32 {

range "0..4294967295";

}

description "It identifies a gNB within a PLMN. The gNB ID is part of

the NR Cell Identifier (NCI) of the gNB cells.";

reference "gNB Identifier (gNB ID) of subclause 8.2 of TS 38.300.

Global gNB ID in subclause 9.3.1.6 of TS 38.413";

}

list tAI {

key nRTAC;

min-elements 1;

max-elements 1;

description "Tracking Area Identity";

reference "subclause 9.3.3.11 in TS 38.413";

uses TAIGrp;

}

}

grouping MappingSetIDBackhaulAddressGrp {

description "Mapping relationship between setID and backhaulAddress of gNB";

leaf idx {

type uint32 ;

description "ID value";

}

leaf setID {

type uint32;

mandatory true;

description "Indicates the setID of gNB.";

reference "Subclause 7.4.1.6 in TS 38.211";

}

list backhaulAddress {

key gNBId;

min-elements 1;

max-elements 1;

description "Indicates the backhauladdress of gNB.";

uses BackhaulAddressGrp;

}

}

grouping GNBCUUPFunctionGrp {

description "Represents the GNBCUUPFunction IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

uses nrrrmpolicy3gpp:RRMPolicy_Grp;

leaf gNBCUUPId {

type uint64 {

range "0..68719476735" ;

}

config false;

mandatory true;

description "Identifies the gNB-CU-UP at least within a gNB-CU-CP";

reference "'gNB-CU-UP ID' in subclause 9.3.1.15 of 3GPP TS 38.463";

}

leaf gNBId {

type uint32;

mandatory true;

description "Identifies a gNB within a PLMN. The gNB ID is part of the

NR Cell Identifier (NCI) of the gNB cells. ";

reference "gNB Identifier (gNB ID) of subclause 8.2 of TS 38.300.

Global gNB ID in subclause 9.3.1.6 of TS 38.413";

}

leaf gNBIdLength {

mandatory true;

type int32 { range "22..32"; }

description "Indicates the number of bits for encoding the gNB Id.";

reference "gNB Id in 3GPP TS 38.300, Global gNB ID in 3GPP TS 38.413";

}

list pLMNInfoList {

description "The PLMNInfoList is a list of PLMNInfo data type. It

defines which PLMNs that can be served by the GNBCUUPFunction and

which S-NSSAIs can be supported by the GNBCUUPFunction for

corresponding PLMN in case of network slicing feature is supported";

key "mcc mnc sd sst";

uses types5g3gpp:PLMNInfo;

}

list mappingSetIDBackhaulAddressList {

key idx;

description "Specifies a list of mappingSetIDBackhaulAddress used to

retrieve the backhaul address of the victim set.

Must be present if Remote Interference Management function is

supported.";

uses MappingSetIDBackhaulAddressGrp;

}

leaf configurable5QISetRef {

type types3gpp:DistinguishedName;

description "DN of the Configurable5QISet that the GNBCUUPFunction

supports (is associated to).";

}

leaf dynamic5QISetRef {

type types3gpp:DistinguishedName;

description "DN of the Dynamic5QISet that the GNBCUUPFunction

supports (is associated to).";

}

}

augment "/me3gpp:ManagedElement" {

list GNBCUUPFunction {

key id;

description "Represents the logical function CU-UP of gNB or en-gNB.";

reference "3GPP TS 28.541";

uses top3gpp:Top_Grp;

container attributes {

uses GNBCUUPFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## E.5.18	module_3gpp-nr-nrm-gnbdufunction.yang

module _3gpp-nr-nrm-gnbdufunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-gnbdufunction";

prefix "gnbdu3gpp";

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-rrmpolicy { prefix nrrrmpolicy3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the GNBDUFunction Information

Object Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2020-10-02 { reference CR-0384 ; }

revision 2020-03-12 { reference "SP-200233 S5-201547" ; }

revision 2020-02-14 { reference S5-20XXXX ; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-08-21 {

description "Initial revision.";

}

feature DRACHOptimizationFunction {

description "Classs representing D-SON function of RACH optimization

feature";

}

grouping GNBDUFunctionGrp {

description "Represents the GNBDUFunction IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

uses nrrrmpolicy3gpp:RRMPolicy_Grp;

leaf gNBId {

type int64 { range "0..4294967295"; }

config false;

mandatory true;

description "Identifies a gNB within a PLMN. The gNB Identifier (gNB ID)

is part of the NR Cell Identifier (NCI) of the gNB cells.";

reference "gNB ID in 3GPP TS 38.300, Global gNB ID in 3GPP TS 38.413";

}

leaf gNBIdLength {

type int32 { range "22..32"; }

mandatory true;

description "Indicates the number of bits for encoding the gNB ID.";

reference "gNB ID in 3GPP TS 38.300, Global gNB ID in 3GPP TS 38.413";

}

leaf gNBDUId {

type int64 { range "0..68719476735"; }

mandatory true;

description "Uniquely identifies the DU at least within a gNB.";

reference "3GPP TS 38.473";

}

leaf gNBDUName {

type string { length "1..150"; }

description "Identifies the Distributed Unit of an NR node";

reference "3GPP TS 38.473";

}

leaf aggressorSetID {

type uint32 { range "0..4194304"; }

config false;

description "Indicates the associated aggressor gNB Set ID of the cell

Valid when Remote Interference Management function is supported.";

reference "3GPP TS 38.211 subclause 7.4.1.6";

}

leaf victimSetID {

type uint32 { range "0..4194304"; }

config false;

description "Indicates the associated victim gNB Set ID of the cell

Valid when Remote Interference Management function is supported.";

reference "3GPP TS 38.211 subclause 7.4.1.6";

}

}

augment "/me3gpp:ManagedElement" {

list GNBDUFunction {

key id;

description "Represents the logical function DU of gNB or en-gNB.";

reference "3GPP TS 28.541";

uses top3gpp:Top_Grp;

container attributes {

uses GNBDUFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## E.5.19	module _3gpp-nr-nrm-nrcellcu.yang

<CODE BEGINS>

module _3gpp-nr-nrm-nrcellcu {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-nrcellcu";

prefix "nrcellcu3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-nr-nrm-gnbcucpfunction { prefix gnbcucp3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the NRCellCU Information Object

Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2021-01-25 { reference CR-0454 ; }

revision 2020-11-25 { reference CR-0386 ; }

revision 2020-11-05 { reference CR-0412 ; }

revision 2020-10-02 { reference CR-0384 ; }

revision 2020-05-08 { reference S5-203316 ; }

revision 2020-02-14 { reference S5-20XXXX ; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 { reference "Initial revision"; }

feature DPCIConfigurationFunction {

description "Class representing Distributed SON or Domain-Centralized SON

function of PCI configuration feature";

}

feature DESManagementFunction {

description "Class representing Distributed SON or Domain-Centralized SON

Energy Saving feature";

}

feature DMROFunction {

description "Class representing D-SON function of MRO feature";

}

feature CESManagementFunction {

description "Class representing Cross Domain-Centralized SON Energy Saving

feature";

}

grouping NRCellCUGrp {

description "Represents the NRCellCU IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

leaf cellLocalId {

description "Identifies an NR cell of a gNB. Together with corresponding

gNB ID it forms the NR Cell Identifier (NCI).";

mandatory true;

type int32 { range "0..16383"; }

}

list pLMNInfoList {

description "The PLMNInfoList is a list of PLMNInfo data type. It defines

which PLMNs that can be served by the NR cell, and which S-NSSAIs that

can be supported by the NR cell for corresponding PLMN in case of

network slicing feature is supported.";

// Note: Whether the attribute pLMNId in the pLMNInfo can be writable

// depends on the implementation.

key "mcc mnc sd sst";

min-elements 1;

uses types5g3gpp:PLMNInfo;

}

leaf nRFrequencyRef {

description "Reference to corresponding NRFrequency instance.";

config false;

type types3gpp:DistinguishedName;

}

}

augment "/me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction" {

list NRCellCU {

description "Represents the information required by CU that is

responsible for the management of inter-cell mobility and neighbour

relations via ANR.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NRCellCUGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

<CODE ENDS>

## E.5.20	module _3gpp-nr-nrm-nrcelldu.yang

<CODE BEGINS>

module _3gpp-nr-nrm-nrcelldu {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-nrcelldu";

prefix "nrcelldu3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-gnbdufunction { prefix gnbdu3gpp; }

import _3gpp-nr-nrm-rrmpolicy { prefix nrrrmpolicy3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the NRCellDU Information Object

Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2021-01-25 { reference CR-0454 ; }

revision 2020-11-25 { reference CR-0386 ; }

revision 2020-11-05 { reference CR-0412 ; }

revision 2020-10-02 { reference CR-0384 ; }

revision 2020-05-08 { reference S5-203316 ; }

revision 2020-02-14 { reference S5-20XXXX ; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-09-03 { reference "Initial revision"; }

feature DRACHOptimizationFunction {

description "Class representing D-SON function of RACH optimization

feature";

}

feature CPCIConfigurationFunction {

description "Class representing Cross Domain-Centralized SON function of

PCI configuration feature";

}

grouping NRCellDUGrp {

description "Represents the NRCellDU IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

uses nrrrmpolicy3gpp:RRMPolicy_Grp;

leaf cellLocalId {

description "Identifies an NR cell of a gNB. Together with the

corresponding gNB identifier in forms the NR Cell Identity (NCI).";

reference "NCI in 3GPP TS 38.300";

mandatory true;

type int32 { range "0..16383"; }

}

leaf operationalState  {

description "Operational state of the NRCellDU instance. Indicates

whether the resource is installed and partially or fully operable

(ENABLED) or the resource is not installed or not operable

(DISABLED).";

config false;

type types3gpp:OperationalState;

}

leaf administrativeState  {

description "Administrative state of the NRCellDU. Indicates the

permission to use or prohibition against using the cell, imposed

through the OAM services.";

type types3gpp:AdministrativeState;

default LOCKED;

}

leaf cellState  {

description "Cell state of the NRCellDU instance. Indicates whether the

cell is not currently in use (IDLE), or currently in use but not

configured to carry traffic (INACTIVE), or currently in use and is

configured to carry traffic (ACTIVE).";

config false;

type types3gpp:CellState;

}

list pLMNInfoList {

description "The PLMNInfoList is a list of PLMNInfo data type. It

defines which PLMNs that can be served by the NR cell, and which

S-NSSAIs that can be supported by the NR cell for corresponding PLMN

in case of network slicing feature is supported. The plMNId of the

first entry of the list is the PLMNId used to construct the nCGI for

the NR cell.";

key "mcc mnc sd sst";

min-elements 1;

ordered-by user;

uses types5g3gpp:PLMNInfo;

}

leaf nRPCI {

description "The Physical Cell Identity (PCI) of the NR cell.";

reference "3GPP TS 36.211";

mandatory true;

type int32 { range "0..1007"; }

}

leaf nRTAC {

description "The common 5GS Tracking Area Code for the PLMNs.";

reference "3GPP TS 23.003, 3GPP TS 38.473";

type types3gpp:Tac;

}

leaf arfcnDL {

description "NR Absolute Radio Frequency Channel Number (NR-ARFCN) for

downlink.";

reference "3GPP TS 38.104";

mandatory true;

type int32;

}

leaf arfcnUL {

description "NR Absolute Radio Frequency Channel Number (NR-ARFCN) for

uplink.";

reference "3GPP TS 38.104";

type int32;

}

leaf arfcnSUL {

description "NR Absolute Radio Frequency Channel Number (NR-ARFCN) for

supplementary uplink.";

reference "3GPP TS 38.104";

type int32;

}

leaf bSChannelBwDL {

description "Base station channel bandwidth for downlink.";

reference "3GPP TS 38.104";

type int32;

units MHz;

}

leaf bSChannelBwUL {

description "Base station channel bandwidth for uplink.";

reference "3GPP TS 38.104";

type int32;

units MHz;

}

leaf bSChannelBwSUL {

description "Base station channel bandwidth for supplementary uplink.";

reference "3GPP TS 38.104";

type int32;

units MHz;

}

leaf ssbFrequency {

description "Indicates cell defining SSB frequency domain position.

Frequency (in terms of NR-ARFCN) of the cell defining SSB transmission.

The frequency identifies the position of resource element RE=#0

(subcarrier #0) of resource block RB#10 of the SS block. The frequency

must be positioned on the NR global frequency raster, as defined in

3GPP TS 38.101-1, and within bSChannelBwDL.";

mandatory true;

type int32 { range "0..3279165"; }

}

leaf ssbPeriodicity {

description "Indicates cell defined SSB periodicity. The SSB periodicity

is used for the rate matching purpose.";

mandatory true;

type int32 { range "5 | 10 | 20 | 40 | 80 | 160"; }

units "subframes (ms)";

}

leaf ssbSubCarrierSpacing {

description "Subcarrier spacing of SSB. Only the values 15 kHz or 30 kHz

(< 6 GHz), 120 kHz or 240 kHz (> 6 GHz) are applicable.";

reference "3GPP TS 38.211";

mandatory true;

type int32 { range "15 | 30 | 120 | 240"; }

units kHz;

}

leaf ssbOffset {

description "Indicates cell defining SSB time domain position. Defined

as the offset of the measurement window, in which to receive SS/PBCH

blocks, where allowed values depend on the ssbPeriodicity

(ssbOffset < ssbPeriodicity).";

mandatory true;

type int32 { range "0..159"; }

units "subframes (ms)";

}

leaf ssbDuration {

description "Duration of the measurement window in which to receive

SS/PBCH blocks.";

reference "3GPP TS 38.213";

mandatory true;

type int32 { range "1..5"; }

units "subframes (ms)";

}

leaf-list nRSectorCarrierRef {

description "Reference to corresponding NRSectorCarrier instance.";

min-elements 1;

type types3gpp:DistinguishedName;

}

leaf-list bWPRef {

description "Reference to corresponding BWP instance.";

type types3gpp:DistinguishedName;

}

leaf-list nRFrequencyRef {

description "Reference to corresponding NRFrequency instance.";

type types3gpp:DistinguishedName;

}

}

augment "/me3gpp:ManagedElement/gnbdu3gpp:GNBDUFunction" {

list NRCellDU {

description "Represents the information of a cell known by DU.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NRCellDUGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

<CODE ENDS>

## E.5.21	module _3gpp-nr-nrm-nrcellrelation.yang

<CODE BEGINS>

module _3gpp-nr-nrm-nrcellrelation {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-nrcellrelation";

prefix "nrcellrel3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-gnbcucpfunction { prefix gnbcucp3gpp; }

import _3gpp-nr-nrm-nrcellcu { prefix nrcellcu3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the NRCellRelation Information

Object Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2021-01-25 { reference CR-0454 ; }

revision 2020-06-03 { reference S5-202333 ; }

revision 2020-04-23 { reference CR0281 ; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-08-30 {

description "Initial revision";

}

typedef EnergySavingCoverage {

type enumeration {

enum FULL;

enum NO;

enum PARTIAL;

}

}

grouping NRCellRelationGrp {

description "Represents the NRCellRelation IOC.";

reference "3GPP TS 28.541";

leaf nRTCI {

description "Target NR Cell Identifier. It consists of NR Cell

Identifier (NCI) and Physical Cell Identifier of the target NR cell

(nRPCI).";

type uint64;

}

container cellIndividualOffset {

description "A set of offset values for the neighbour cell. Used when

UE is in connected mode. Defined for rsrpOffsetSSB, rsrqOffsetSSB,

sinrOffsetSSB, rsrpOffsetCSI-RS, rsrqOffsetCSI-RS and

sinrOffsetCSI-RS.";

reference "cellIndividualOffset in MeasObjectNR in 3GPP TS 38.331";

leaf rsrpOffsetSsb {

description "Offset value of rsrpOffsetSSB.";

default 0;

type types3gpp:QOffsetRange;

}

leaf rsrqOffsetSsb{

description "Offset value of rsrqOffsetSSB.";

default 0;

type types3gpp:QOffsetRange;

}

leaf sinrOffsetSsb {

description "Offset value of sinrOffsetSSB.";

default 0;

type types3gpp:QOffsetRange;

}

leaf rsrpOffsetCsiRs{

description "Offset value of rsrpOffsetCSI-RS.";

default 0;

type types3gpp:QOffsetRange;

}

leaf rsrqOffsetCsiRs {

description "Offset value of rsrqOffsetCSI-RS.";

default 0;

type types3gpp:QOffsetRange;

}

leaf sinrOffsetCsiRs {

description "Offset value of sinrOffsetCSI-RS.";

default 0;

type types3gpp:QOffsetRange;

}

}

leaf nRFreqRelationRef {

description "Reference to a corresponding NRFreqRelation instance.";

mandatory true;

type types3gpp:DistinguishedName;

}

leaf adjacentNRCellRef {

description "Reference to an adjacent NR cell (NRCellCU or

ExternalNRCellCU).";

mandatory true;

type types3gpp:DistinguishedName;

}

leaf isRemoveAllowed {

type boolean;

default true;

description "True if the ANR function in the node is allowed to remove this relation.";

}

leaf isHOAllowed {

type boolean;

default true;

description "True if handovers are allowed over this relation.";

}

leaf isESCoveredBy {

description "Indicates whether the adjacent cell

provides no, partial or full coverage for the parent cell

instance. Adjacent cells with this attribute equal to FULL are

recommended to be considered as candidate cells to take over the

coverage when the original cell is about to be changed to energy

saving state. All adjacent cells with this property equal

to PARTIAL are recommended to be considered as entirety of candidate

cells to take over the coverage when the original cell is about to be

changed to energy saving state.";

type EnergySavingCoverage;

}

}

augment /me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction/nrcellcu3gpp:NRCellCU {

list NRCellRelation {

description "Represents a neighbour cell relation from a source cell

to a target cell, where the target cell is an NRCellCU or

ExternalNRCellCU instance.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NRCellRelationGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

<CODE ENDS>

## E.5.22	module _3gpp-nr-nrm-nrfreqrelation@2019-10-28.yang

module _3gpp-nr-nrm-nrfreqrelation {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-nrfreqrelation";

prefix "nrfreqrel3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-gnbcucpfunction { prefix gnbcucp3gpp; }

import _3gpp-nr-nrm-nrcellcu { prefix nrcellcu3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the NRFreqRelation Information

Object Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2020-04-23 { reference CR0281 ; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 {

description "Initial revision";

}

grouping NRFreqRelationGrp {

description "Represents the NRFreqRelation IOC.";

reference "3GPP TS 28.541";

container offsetMO {

description "A set of offset values applicable to all measured cells

with reference signal(s) indicated in corresponding MeasObjectNR. It

is used to indicate a cell, beam or measurement object specific offset

to be applied when evaluating candidates for cell re-selection or when

evaluating triggering conditions for measurement reporting. It is

defined for rsrpOffsetSSB, rsrqOffsetSSB, sinrOffsetSSB,

rsrpOffsetCSI-RS, rsrqOffsetCSI-RS and sinrOffsetCSI-RS.";

reference "offsetMO in MeasObjectNR in 3GPP TS 38.331";

leaf rsrpOffsetSsb {

description "Offset value of rsrpOffsetSSB.";

default 0;

type types3gpp:QOffsetRange;

}

leaf rsrqOffsetSsb {

description "Offset value of rsrqOffsetSSB.";

default 0;

type types3gpp:QOffsetRange;

}

leaf sinrOffsetSsb {

description "Offset value of sinrOffsetSSB.";

default 0;

type types3gpp:QOffsetRange;

}

leaf rsrpOffsetCsiRs {

description "Offset value of rsrpOffsetCSI-RS.";

default 0;

type types3gpp:QOffsetRange;

}

leaf rsrqOffsetCsiRs {

description "Offset value of rsrqOffsetCSI-RS.";

default 0;

type types3gpp:QOffsetRange;

}

leaf sinrOffsetCsiRs {

description "Offset value of sinrOffsetCSI-RS.";

default 0;

type types3gpp:QOffsetRange;

}

}

leaf-list blackListEntry {

description "A list of Physical Cell Identities (PCIs) that are

blacklisted in NR measurements.";

reference "3GPP TS 38.331";

min-elements 0;

type uint16 { range "0..1007"; }

}

leaf-list blackListEntryIdleMode {

description "A list of Physical Cell Identities (PCIs) that are

blacklisted in SIB4 and SIB5.";

min-elements 0;

type uint16 { range "0..1007"; }

}

leaf cellReselectionPriority {

description "The absolute priority of the carrier frequency used by the

cell reselection procedure. Value 0 means lowest priority. The value

must not already used by other RAT, i.e. equal priorities between RATs

are not supported. The UE behaviour when no value is entered is

specified in subclause 5.2.4.1 of 3GPP TS 38.304.";

reference "CellReselectionPriority in 3GPP TS 38.331, priority in

3GPP TS 38.304";

type uint32;

default 0;

}

leaf cellReselectionSubPriority {

description "Indicates a fractional value to be added to the value of

cellReselectionPriority to obtain the absolute priority of the

concerned carrier frequency for E-UTRA and NR.";

reference "3GPP TS 38.331";

type uint8 { range "2 | 4 | 6 | 8"; }

units "0.1";

}

leaf pMax {

description "Used for calculation of the parameter Pcompensation

(defined in 3GPP TS 38.304), at cell reselection to a cell.";

reference "PEMAX in 3GPP TS 38.101-1";

mandatory false;

type int32 { range "-30..33"; }

units dBm;

}

leaf qOffsetFreq {

description "The frequency specific offset applied when evaluating

candidates for cell reselection.";

mandatory false;

type types3gpp:QOffsetRange;

default 0;

}

leaf qQualMin {

description "Indicates the minimum required quality level in the cell.

Value 0 means that it is not sent and UE applies in such case the

(default) value of negative infinity for Qqualmin. Sent in SIB3 or

SIB5.";

reference "3GPP TS 38.304";

type int32 { range "-34..-3 | 0"; }

units dB;

default 0;

}

leaf qRxLevMin {

description "Indicates the required minimum received Reference Symbol

Received Power (RSRP) level in the NR frequency for cell reselection.

Broadcast in SIB3 or SIB5, depending on whether the related frequency

is intra- or inter-frequency. Resolution is 2.";

reference "3GPP TS 38.304";

mandatory true;

type int32 { range "-140..-44"; }

units dBm;

}

leaf threshXHighP {

description "Specifies the Srxlev threshold used by the UE when

reselecting towards a higher priority RAT/frequency than the current

serving frequency. Each frequency of NR and E-UTRAN might have a

specific threshold. Resolution is 2.";

reference "ThreshX, HighP in 3GPP TS 38.304";

mandatory true;

type int32 { range "0..62"; }

units dB;

}

leaf threshXHighQ {

description "Specifies the Squal threshold used by the UE when

reselecting towards a higher priority RAT/frequency than the current

serving frequency. Each frequency of NR and E-UTRAN might have a

specific threshold.";

reference "ThreshX, HighQ in 3GPP TS 38.304";

mandatory true;

type int32 { range "0..31"; }

units dB;

}

leaf threshXLowP {

description "Specifies the Srxlev threshold used by the UE when

reselecting towards a lower priority RAT/frequency than the current

serving frequency. Each frequency of NR and E-UTRAN might have a

specific threshold. Resolution is 2.";

reference "ThreshX, LowP in 3GPP TS 38.304";

mandatory true;

type int32 { range "0..62"; }

units dB;

}

leaf threshXLowQ {

description "Specifies the Squal threshold used by the UE when

reselecting towards a lower priority RAT/frequency than the current

serving frequency. Each frequency of NR and E-UTRAN might have a

specific threshold.";

reference "ThreshX, LowQ in 3GPP TS 38.304";

mandatory true;

type int32 { range "0..31"; }

units dB;

}

leaf tReselectionNR {

description "Cell reselection timer for NR.";

reference "TreselectionRAT for NR in 3GPP TS 38.331";

mandatory true;

type int32 { range "0..7"; }

units s;

}

leaf tReselectionNRSfHigh {

description "The attribute tReselectionNr (parameter TreselectionNR in

3GPP TS 38.304) is multiplied with this scaling factor if the UE is

in high mobility state.";

reference "Speed dependent ScalingFactor for TreselectionNR for high

mobility state in 3GPP TS 38.304";

mandatory true;

type uint8 { range "25 | 50 | 75 | 100"; }

units %;

}

leaf tReselectionNRSfMedium {

description "The attribute tReselectionNr (parameter TreselectionNR in

3GPP TS 38.304) multiplied with this scaling factor if the UE is in

medium mobility state.";

reference "Speed dependent ScalingFactor for TreselectionNR for medium

mobility state in 3GPP TS 38.304";

mandatory true;

type uint8 { range "25 | 50 | 75 | 100"; }

units %;

}

leaf nRFrequencyRef {

description "Reference to a corresponding NRFrequency instance.";

mandatory true;

type types3gpp:DistinguishedName;

}

}

augment /me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction/nrcellcu3gpp:NRCellCU {

list NRFreqRelation {

description "Together with the target NRFrequency, it represents the

frequency properties applicable to the referencing NRFreqRelation.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NRFreqRelationGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## E.5.23	module _3gpp-nr-nrm-nrfrequency@2019-10-28.yang

module _3gpp-nr-nrm-nrfrequency {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-nrnetwork-nrfrequency";

prefix "nrfreq3gpp";

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-nr-nrm-nrnetwork { prefix nrnet3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the NRFrequency Information Object

Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 {

description "Initial revision";

}

grouping NRFrequencyGrp {

description "Represents the NRFrequency IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

leaf absoluteFrequencySSB {

description "The absolute frequency applicable for a downlink NR carrier

frequency associated with the SSB, in terms of NR-ARFCN.";

mandatory true;

type uint32 { range "0.. 3279165"; }

}

leaf sSBSubCarrierSpacing {

description "Sub-carrier spacing of the SSB.";

mandatory true;

type uint8 { range "15 | 30 | 60 | 120"; }

units "kHz";

}

leaf-list multiFrequencyBandListNR {

description "List of additional frequency bands the frequency belongs to.

The list is automatically set by the gNB.";

config false;

min-elements 0;

type uint16 { range "1..256"; }

}

}

grouping NRFrequencyWrapper {

list NRFrequency {

description "Represents certain NR frequency properties.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NRFrequencyGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

augment "/subnet3gpp:SubNetwork" {

if-feature subnet3gpp:ExternalsUnderSubNetwork ;

uses NRFrequencyWrapper;

}

augment "/nrnet3gpp:NRNetwork" {

if-feature nrnet3gpp:ExternalsUnderNRNetwork;

uses NRFrequencyWrapper;

}

}

## E.5.24	module _3gpp-nr-nrm-nrnetwork@2019-06-17.yang

module _3gpp-nr-nrm-nrnetwork {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-nrnetwork";

prefix "nrnet3gpp";

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

description "Defines the YANG mapping of the NRNetwork Information Object

Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2019-06-17 {

description "Initial revision";

}

feature ExternalsUnderNRNetwork {

description "Classes representing external entities like NRFrequency,

ExternalGNBCUCPFunction, ExternalGNBDUFunction

are contained under a NRNetwork list/class.";

}

grouping NRNetworkGrp {

description "Represents the NRNetwork IOC.";

reference "3GPP TS 28.541";

uses subnet3gpp:SubNetworkGrp;

}

list NRNetwork {

description "A subnetwork containing gNB external NR entities.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NRNetworkGrp;

}

}

}

## E.5.25	module _3gpp-nr-nrm-nrsectorcarrier.yang

module _3gpp-nr-nrm-nrsectorcarrier {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-nrnetwork-nrsectorcarrier";

prefix "nrsectcarr3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-nr-nrm-gnbdufunction { prefix gnbdu3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the NRSectorCarrier Information

Object Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2020-05-28 { reference CR-0316 ; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 {

description "Initial revision";

}

grouping NRSectorCarrierGrp {

description "Represents the NRSectorCarrier IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

leaf txDirection {

description "Indicates if the transmission direction is downlink,

uplink, or both downlink and uplink.";

mandatory true;

type types3gpp:TxDirection;

}

leaf configuredMaxTxPower {

description "Maximum transmisssion power at the antenna port for all

downlink channels, used simultaneously in a cell, added together.

Condition:  The sector-carrier has a downlink and the

configuration of Tx power at antenna port reference point is supported.";

mandatory true;

type int32;

units mW;

}

leaf configuredMaxTxEIRP {

type int64;

units dBm;

mandatory true;

description "The maximum emitted isotroptic radiated power (EIRP) in dBm

for all downlink channels, used simultaneously in a cell, added together.

Condition: the sector-carrier has a downlink and the

configuration of emitted isotropic radiated power is supported";

}

leaf arfcnDL {

description "NR Absolute Radio Frequency Channel Number (NR-ARFCN)

for downlink.

Condition:  The sector-carrier has a downlink AND the value

differs from the referring cell's value of arfcnDL.";

reference "3GPP TS 38.104";

mandatory true;

type int32 { range "0..3279165"; }

}

leaf arfcnUL {

description "NR Absolute Radio Frequency Channel Number (NR-ARFCN)

for uplink.

Condition: The sector-carrier has an uplink AND the value

differs from the referring cell's value of arfcnUL.";

reference "3GPP TS 38.104";

mandatory true;

type int32 { range "0..3279165"; }

}

leaf bSChannelBwDL {

description "Base station channel bandwitdth for downlink.

Condition: The sector-carrier has a downlink AND the value

differs from the referring cell's value of bSChannelBwDL.";

reference "3GPP TS 38.104";

mandatory true;

type int32 { range "5 | 10 | 15 | 20 | 30 | 40 | 50 | 60 | 70 | 80 |

90 | 100"; }

units MHz;

}

leaf bSChannelBwUL {

description "Base station channel bandwitdth for uplink.";

reference "3GPP TS 38.104";

mandatory true;

type int32 { range "5 | 10 | 15 | 20 | 30 | 40 | 50 | 60 | 70 | 80 |

90 | 100"; }

units MHz;

}

leaf sectorEquipmentFunctionRef {

description "Reference to corresponding SectorEquipmentFunction

instance.";

reference "3GPP TS 23.622";

mandatory true;

type types3gpp:DistinguishedName;

}

}

augment "/me3gpp:ManagedElement/gnbdu3gpp:GNBDUFunction" {

list NRSectorCarrier {

description "Represents the resources of each transmission point

included in the cell.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NRSectorCarrierGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## E.5.26	module _3gpp-nr-nrm-rrmpolicy.yang

module _3gpp-nr-nrm-rrmpolicy {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-rrmpolicy";

prefix "nrrrmpolicy3gpp";

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the RRMPolicy abstract class that

is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2020-11-05 { reference CR-0412 ; }

revision 2020-04-28 { reference "CR0285"; }

revision 2020-02-14 { reference "Initial revision"; }

grouping rRMPolicyMemberGrp {

description "This data type represents an RRM Policy member that will be

part of a rRMPolicyMemberList. A RRMPolicyMember is defined by its

pLMNId and sNSSAI (S-NSSAI).

The members in a rRMPolicyMemberList are assigned a specific amount of

RRM resources based on settings in RRMPolicy.";

uses types5g3gpp:PLMNInfo;

}

typedef CyclicPrefix {

type enumeration {

enum NORMAL;

enum EXTENDED;

}

}

grouping RRMPolicy_Grp {

description "This IOC represents the properties of an abstract RRMPolicy

. The RRMPolicy_ IOC needs to be subclassed to be instantiated.

It defines two attributes apart from those inherited from Top IOC, the

resourceType attribute defines type of resource (PRB, RRC

connected users, DRB usage etc.) and the rRMPolicyMemberList attribute

defines the RRMPolicyMember(s)that are subject to this policy.

An RRM resource (defined in resourceType

attribute) is located in NRCellDU, NRCellCU, GNBDUFunction,

GNBCUCPFunction or in GNBCUUPFunction. The RRMPolicyRatio IOC is one

realization of a RRMPolicy_ IOC. This RRM framework allows adding new

policies, both standardized (like RRMPolicyRatio) or as vendor specific,

by inheriting from the abstract RRMPolicy_ IOC.";

leaf resourceType {

description "The resourceType attribute defines type of resource (PRB,

RRC connected users, DRB usage etc.) that is subject to policy.

Valid values are 'PRB', 'RRC' or 'DRB'";

mandatory true;

type string;

}

list rRMPolicyMemberList{

description "It represents the list of RRMPolicyMember (s) that the

managed object is supporting. A RRMPolicyMember <<dataType>> include

the PLMNId <<dataType>> and S-NSSAI <<dataType>>." ;

min-elements 1;

key "mcc mnc sd sst";

uses rRMPolicyMemberGrp;

}

} // grouping

grouping RRMPolicyRatioGrp {

description "Represents the RRMPolicyRatio concrete IOC.";

uses RRMPolicy_Grp;    // Inherits RRMPolicy_

leaf rRMPolicyMaxRatio {

description " This attribute specifies the maximum percentage of radio

resource that can be used by the associated rRMPolicyMemberList.

The maximum percentage of radio resource include at least one of

the shared resources, prioritized resources and dedicated resources.

The sum of the rRMPolicyMaxRatio values assigned to all RRMPolicyRatio(s)

name-contained by same ManagedEntity can be greater that 100.";

default 100;

type uint8 { range "0..100"; }

units percent;

}

leaf rRMPolicyMinRatio {

description " This attribute specifies the minimum percentage of radio

resources that can be used by the associated rRMPolicyMemberList.

The minimum percentage of radio resources including at least one of

prioritized resources and dedicated resources. The sum of the

rRMPolicyMinRatio values assigned to all RRM PolicyRatio(s)

name-contained by same ManagedEntity shall be less or equal 100.";

default 0;

type uint8 { range "0..100"; }

units percent;

}

leaf rRMPolicyDedicatedRatio {

description " This attribute specifies the percentage of radio resource

that dedicatedly used by the associated rRMPolicyMemberList. The sum of

the rRMPolicyDeidctaedRatio values assigned to all RRMPolicyRatio(s)

name-contained by same ManagedEntity shall be less or equal 100. ";

default 0;

type uint8 { range "0..100"; }

units percent;

}

}

list RRMPolicyRatio {

description " The RRMPolicyRatio IOC is one realization of a RRMPolicy_ IOC,

see the inheritance in Figure 4.2.1.2-1. This RRM framework allows

adding new policies, both standardized (like RRMPolicyRatio) or as

vendor specific, by inheriting from the

abstract RRMPolicy_ IOC. For details see subclause 4.3.36.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses RRMPolicyRatioGrp;

}

}

}

## E.5.27	Void

## E.5.28	module _3gpp-nr-nrm-danrmanagementfunction.yang

module _3gpp-nr-nrm-danrmanagementfunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-danrmanagementfunction";

prefix "danrmanagementfunction3gpp";

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-gnbcucpfunction { prefix gnbcucp3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the DANRManagementFunction Information Object Class

(IOC) that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2020-05-08 { reference S5-203316; }

grouping DANRManagementFunctionGrp {

description "Represents the DANRManagementFunction IOC.";

reference "3GPP TS 28.541";

uses top3gpp:Top_Grp;

leaf intrasystemANRManagementSwitch {

description "This attribute determines whether the intra-system ANR function is activated or deactivated.";

type boolean;

}

leaf intersystemANRManagementSwitch {

description "This attribute determines whether the inter-system ANR function is activated or deactivated.";

type boolean;

}

}

augment "/me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction" {

if-feature gnbcucp3gpp:DANRManagementFunction;

uses DANRManagementFunctionGrp;

}

}

## E.5.29	module _3gpp-nr-nrm-desmanagementfunction.yang

module _3gpp-nr-nrm-desmanagementfunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-desmanagementfunction";

prefix "desmanagementfunction3gpp";

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-gnbcucpfunction { prefix gnbcucp3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-nr-nrm-nrcellcu { prefix nrcellcu3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the DESManagementFunction Information Object Class

(IOC) that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2020-05-08 { reference S5-203316; }

grouping DESManagementFunctionGrp {

description "Represents the DESManagementFunction IOC.";

reference "3GPP TS 28.541";

uses top3gpp:Top_Grp;

leaf desSwitch {

description "This attribute determines whether the Distributed SON or Domain-Centralized SON energy saving function is enabled or disabled.";

type boolean;

}

list intraRatEsActivationOriginalCellLoadParameters {

key loadThreshold;

description "This attributes is relevant, if the cell acts as an original cell.This attribute indicates the traffic load threshold and the time duration, which are used by distributed ES algorithms to allow a cell to enter the energySaving state.";

leaf loadThreshold {type int32;}

container attributes {

uses IntraRatEsActivationOriginalCellLoadParametersGrp;

}

}

list intraRatEsActivationCandidateCellsLoadParameters {

key loadThreshold;

description "This attribute indicates the traffic load threshold and the time duration, which are used by distributed ES algorithms level to allow a n ‘original’ cell to enter the energySaving state.";

leaf loadThreshold {type int32;}

container attributes {

uses IntraRatEsActivationCandidateCellsLoadParametersGrp;

}

}

list intraRatEsDeactivationCandidateCellsLoadParameters {

key loadThreshold;

description "This attributes is relevant, if the cell acts as a candidate cell.This attribute indicates the traffic load threshold  and the time duration which is used by distributed ES algorithms to allow a cell to leave the energySaving state.";

leaf loadThreshold {type int32;}

container attributes {

uses IntraRatEsDeactivationCandidateCellsLoadParametersGrp;

}

}

list esNotAllowedTimePeriod {

key startTimeandendTime;

description "This attribute indicates a list of time periods during which inter-RAT energy saving is not allowed.";

leaf startTimeandendTime {type string;}

container attributes {

uses EsNotAllowedTimePeriodGrp;

}

}

list interRatEsActivationOriginalCellParameters {

key loadThreshold;

description "This attribute indicates the traffic load threshold and the time duration, which are used by distributed inter-RAT ES algorithms to allow an original cell to enter the energySaving state.";

leaf loadThreshold {type int32;}

container attributes {

uses InterRatEsActivationOriginalCellParametersGrp;

}

}

list interRatEsActivationCandidateCellParameters {

key loadThreshold;

description "This attribute indicates the traffic load threshold and the time duration, which are used by distributed inter-RAT ES algorithms to allow an original cell to enter the energySaving state.";

leaf loadThreshold {type int32;}

container attributes {

uses InterRatEsActivationCandidateCellParametersGrp;

}

}

list interRatEsDeactivationCandidateCellParameters {

key loadThreshold;

description "This attribute indicates the traffic load threshold and the time duration which is used by distributed inter-RAT ES algorithms to allow an original cell to leave the energySaving state.";

leaf loadThreshold {type int32;}

container attributes {

uses InterRatEsDeactivationCandidateCellParametersGrp;

}

}

leaf energySavingState {

description "Specifies the status regarding the energy saving in the cell.";

type enumeration {

enum isNotEnergySaving;

enum isEnergySaving;

}

}

leaf isProbingCapable {

description " This attribute indicates whether this cell is capable of performing the ES probing procedure.";

type enumeration{

enum yes;

enum no;

}

}

}

grouping IntraRatEsActivationOriginalCellLoadParametersGrp {

description "Represents the the traffic load threshold and the time duration.";

leaf loadThreshold {

description "This attribute is used by distributed ES algorithms to allow a cell to enter the energySaving state.";

type int32 { range "0..10000"; }

units "1";

}

leaf timeDuration {

description " The time duration indicates how long the load needs to have been below the threshold.";

type int32 { range "0..900"; }

units "1";

}

}

grouping IntraRatEsActivationCandidateCellsLoadParametersGrp {

description "Represents the the traffic load threshold and the time duration.";

leaf loadThreshold {

description "This attribute is used by distributed ES algorithms to allow a cell to enter the energySaving state.";

type int32 { range "0..10000"; }

units "1";

}

leaf timeDuration {

description " The time duration indicates how long the load needs to have been below the threshold.";

type int32 { range "0..900"; }

units "1";

}

}

grouping IntraRatEsDeactivationCandidateCellsLoadParametersGrp {

description "Represents the the traffic load threshold and the time duration.";

leaf loadThreshold {

description "This attribute is used by distributed ES algorithms to allow a cell to enter the energySaving state.";

type int32 { range "0..10000"; }

units "1";

}

leaf timeDuration {

description " The time duration indicates how long the load needs to have been below the threshold.";

type int32 { range "0..900"; }

units "1";

}

}

grouping EsNotAllowedTimePeriodGrp {

description "Represents the the traffic load threshold and the time duration.";

leaf startTimeandendTime {

description "This field indicate valid UTC time.";

type string;

}

leaf periodOfDay {

description "This field indicate the period of day.";

type string;

}

leaf daysOfWeekList {

description "This field indicate the list of weekday.";

type string;

}

leaf listoftimeperiods {

description "This field indicate the list of time periods.";

type string;

}

}

grouping InterRatEsActivationOriginalCellParametersGrp {

description "Represents the the traffic load threshold and the time duration.";

leaf loadThreshold {

description "The time duration indicates how long the traffic load (both for UL and DL) needs to have been below the threshold.";

type int32 { range "0..10000"; }

units "1";

}

leaf timeDuration {

description " The time duration indicates how long the load needs to have been below the threshold.";

type int32 { range "0..900"; }

units "1";

}

}

grouping InterRatEsActivationCandidateCellParametersGrp {

description "Represents the the traffic load threshold and the time duration.";

leaf loadThreshold {

description "This attribute is used by distributed ES algorithms to allow a cell to enter the energySaving state.";

type int32 { range "0..10000"; }

units "1";

}

leaf timeDuration {

description "The time duration indicates how long the traffic load (both for UL and DL) in the candidate cell needs to have been below the threshold before any original cells which will be provided backup coverage by the candidate cell enters energySaving state.";

type int32 { range "0..900"; }

units "1";

}

}

grouping InterRatEsDeactivationCandidateCellParametersGrp {

description "Represents the the traffic load threshold and the time duration.";

leaf loadThreshold {

description "This attribute is used by distributed ES algorithms to allow a cell to enter the energySaving state.";

type int32 { range "0..10000"; }

units "1";

}

leaf timeDuration {

description "The time duration indicates how long the traffic load (either for UL or DL) in the candidate cell needs to have been above the threshold to wake up one or more original cells which have been provided backup coverage by the candidate cell.";

type int32 { range "0..900"; }

units "1";

}

}

augment "/me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction/nrcellcu3gpp:NRCellCU" {

if-feature nrcellcu3gpp:DESManagementFunction;

uses DESManagementFunctionGrp;

}

augment "/me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction" {

if-feature gnbcucp3gpp:DESManagementFunction;

uses DESManagementFunctionGrp;

}

augment "/me3gpp:ManagedElement" {

if-feature me3gpp:DESManagementFunction;

uses DESManagementFunctionGrp;

}

augment "/subnet3gpp:SubNetwork" {

if-feature subnet3gpp:DESManagementFunction;

uses DESManagementFunctionGrp;

}

}

## E.5.30	module _3gpp-nr-nrm-drachoptimizationfunction.yang

<CODE BEGINS>

module _3gpp-nr-nrm-drachoptimizationfunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-drachoptimizationfunction";

prefix "drachoptimizationfunction3gpp";

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-nr-nrm-nrcelldu { prefix nrcelldu3gpp; }

import _3gpp-nr-nrm-gnbdufunction { prefix gnbdu3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the DRACHOptimizationFunction Information Object Class

(IOC) that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2021-01-25 { reference CR-0454 ; }

revision 2020-10-02 { reference "CR-0384, CR-0382" ; }

revision 2020-05-08 { reference S5-203316; }

grouping DRACHOptimizationFunctionGrp {

description "Represents the DRACHOptimizationFunction IOC.";

reference "3GPP TS 28.541";

uses top3gpp:Top_Grp;

list ueAccProbilityDist {

key targetProbability;

description "This is a list of target Access Probability (APn) for the RACH optimization function.";

leaf targetProbability {type TargetProbability;}

container attributes {

uses UeAccProbilityDistGrp;

}

}

list ueAccDelayProbilityDist {

key targetProbability;

description "This is a list of target Access Delay probability (ADP) for the RACH optimization function.";

leaf targetProbability {type TargetProbability;}

container attributes {

uses UeAccDelayProbilityDistGrp;

}

}

leaf drachOptimizationControl {

description "This attribute determines whether the RACH Optimization function is enabled or disabled.";

type boolean;

}

}

typedef TargetProbability {

type enumeration  {

enum 25;

enum 50;

enum 75;

enum 90;

}

}

typedef Numberofpreamblessent {

type int32  { range "1..200"; }

units "1";

}

typedef Accessdelay {

type int32  { range "10..560"; }

units "1";

}

grouping UeAccProbilityDistGrp {

description "Represents the target Access Probability (APn) for the RACH optimization function.";

leaf targetProbability {

description "This attribute determines the target Probability.";

mandatory true;

type TargetProbability;

}

leaf numberofpreamblessent {

description "This attribute determines the number of preambles sent.";

mandatory true;

type Numberofpreamblessent;

}

}

grouping UeAccDelayProbilityDistGrp {

description "Represents the target Access Delay probability (ADP) for the RACH optimization function.";

leaf targetProbability {

description "This attribute determines the target Probability.";

mandatory true;

type TargetProbability;

}

leaf accessdelay {

description "This attribute determines the access delay.";

mandatory true;

type Accessdelay;

}

}

augment "/me3gpp:ManagedElement/gnbdu3gpp:GNBDUFunction/nrcelldu3gpp:NRCellDU" {

if-feature nrcelldu3gpp:DRACHOptimizationFunction;

uses DRACHOptimizationFunctionGrp;

}

augment "/me3gpp:ManagedElement/gnbdu3gpp:GNBDUFunction" {

if-feature gnbdu3gpp:DRACHOptimizationFunction;

uses DRACHOptimizationFunctionGrp;

}

augment "/me3gpp:ManagedElement" {

if-feature me3gpp:DRACHOptimizationFunction;

uses DRACHOptimizationFunctionGrp;

}

augment "/subnet3gpp:SubNetwork" {

if-feature nrcelldu3gpp:DRACHOptimizationFunction;

uses DRACHOptimizationFunctionGrp;

}

}

<CODE ENDS>

## E.5.31	module _3gpp-nr-nrm-dmrofunction.yang

module _3gpp-nr-nrm-dmrofunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-dmrofunction";

prefix "dmrofunction3gpp";

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-gnbcucpfunction { prefix gnbcucp3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-nr-nrm-nrcellcu { prefix nrcellcu3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the DMROFunction Information Object Class

(IOC) that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2020-05-08 { reference S5-203316; }

grouping DMROFunctionGrp {

description "Represents the DMROFunction IOC.";

reference "3GPP TS 28.541";

uses top3gpp:Top_Grp;

leaf maximumDeviationHoTrigger {

description "This parameter defines the maximum allowed absolute deviation of the Handover Trigger, from the default point of operation.";

type int32 { range "-20..20"; }

units "0.5";

}

leaf minimumTimeBetweenHoTriggerChange {

description "This parameter defines the minimum allowed time interval between two Handover Trigger change performed by MRO. This is used to control the stability and convergence of the algorithm.";

type int32 { range "0..604800"; }

units "1";

}

leaf tstoreUEcntxt {

description "The timer used for detection of too early HO, too late HO and HO to wrong cell.";

type int32 { range "0..1023"; }

units "100";

}

leaf dmroControl {

description " This attribute determines whether the MRO function is enabled or disabled.";

type boolean;

}

}

augment "/me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction/nrcellcu3gpp:NRCellCU" {

if-feature nrcellcu3gpp:DMROFunction;

uses DMROFunctionGrp;

}

augment "/me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction" {

if-feature gnbcucp3gpp:DMROFunction;

uses DMROFunctionGrp;

}

augment "/me3gpp:ManagedElement" {

if-feature me3gpp:DMROFunction;

uses DMROFunctionGrp;

}

augment "/subnet3gpp:SubNetwork" {

if-feature subnet3gpp:DMROFunction;

uses DMROFunctionGrp;

}

}

## E.5.32	module _3gpp-nr-nrm-dpciconfigurationfunction.yang

<CODE BEGINS>

module _3gpp-nr-nrm-dpciconfigurationfunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-dpciconfigurationfunction";

prefix "dpciconfigurationfunction3gpp";

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-nrcellcu { prefix nrcellcu3gpp; }

import _3gpp-nr-nrm-gnbcucpfunction { prefix gnbcucp3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the DPCIConfigurationFunction Information Object Class

(IOC) that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2021-01-25 { reference CR-0454 ; }

revision 2020-11-25 { reference CR-0386 ; }

revision 2020-05-08 { reference S5-203316; }

grouping DPCIConfigurationFunctionGrp {

description "Represents the DPCICONFIGURATIONFunction IOC.";

reference "3GPP TS 28.541";

uses top3gpp:Top_Grp;

list nRPciList {

key NRPci;

description "This holds a list of physical cell identities that can be assigned to the NR cells. This attribute shall be supported if D-SON PCI configuration or domain Centralized SON PCI configuration function is supported.";

leaf NRPci {type int32;}

container attributes {

uses NRPciListGrp;

}

}

leaf dPciConfigurationControl {

description " This attribute determines whether the Distributed SON or Domain-Centralized SON PCI configuration Function is enabled or disabled.";

type boolean;

}

}

grouping NRPciListGrp {

description "Represents the NR PCI list for the PCI configuration function.";

leaf NRPci {

description "This attribute determines the NR PCI.";

type int32 { range "0..1007"; }

units "1";

}

}

augment "/me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction/nrcellcu3gpp:NRCellCU" {

if-feature nrcellcu3gpp:DPCIConfigurationFunction;

uses DPCIConfigurationFunctionGrp;

}

augment "/me3gpp:ManagedElement" {

if-feature me3gpp:DPCIConfigurationFunction;

uses DPCIConfigurationFunctionGrp;

}

augment "/subnet3gpp:SubNetwork" {

if-feature subnet3gpp:DPCIConfigurationFunction;

uses DPCIConfigurationFunctionGrp;

}

}

<CODE ENDS>

## E.5.33	module _3gpp-nr-nrm-cpciconfigurationfunction.yang

module _3gpp-nr-nrm-cpciconfigurationfunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-cpciconfigurationfunction";

prefix "cpciconfigurationfunction3gpp";

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-nrcelldu { prefix nrcelldu3gpp; }

import _3gpp-nr-nrm-gnbdufunction { prefix gnbdu3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the CPCIConfigurationFunction Information Object Class

(IOC) that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2020-05-08 { reference S5-203316; }

grouping CPCIConfigurationFunctionGrp {

description "Represents the CPCICONFIGURATIONFunction IOC.";

reference "3GPP TS 28.541";

uses top3gpp:Top_Grp;

list cSonPciList {

key NRPci;

description " This holds a list of physical cell identities that can be assigned to the pci attribute by gNB. The assignment algorithm is not specified. This attribute shall be supported if and only if the C-SON PCI configuration is supported.";

leaf NRPci {type int32;}

container attributes {

uses CSonPciListGrp;

}

}

leaf cPciConfigurationControl {

description "This attribute determines whether the Cross Domain-Centralized SON PCI configuration function is enabled or disabled.";

type boolean;

}

}

grouping CSonPciListGrp {

description "Represents the C-SON PCI list for the PCI configuration function.";

leaf NRPci {

description "This attribute determines the NR PCI.";

type int32 { range "0..1007"; }

units "1";

}

}

augment "/me3gpp:ManagedElement/gnbdu3gpp:GNBDUFunction/nrcelldu3gpp:NRCellDU" {

if-feature nrcelldu3gpp:CPCIConfigurationFunction;

uses CPCIConfigurationFunctionGrp;

}

augment "/me3gpp:ManagedElement" {

if-feature me3gpp:CPCIConfigurationFunction;

uses CPCIConfigurationFunctionGrp;

}

augment "/subnet3gpp:SubNetwork" {

if-feature subnet3gpp:CPCIConfigurationFunction;

uses CPCIConfigurationFunctionGrp;

}

}

## E.5.34	module _3gpp-nr-nrm-cesmanagementfunction.yang

module _3gpp-nr-nrm-cesmanagementfunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-cesmanagementfunction";

prefix "cesmanagementfunction3gpp";

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-nrcellcu { prefix nrcellcu3gpp; }

import _3gpp-nr-nrm-gnbcucpfunction { prefix gnbcucp3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the CESManagementFunction Information Object Class

(IOC) that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2020-05-08 { reference S5-203316; }

grouping CESManagementFunctionGrp {

description "Represents the CESManagementFunction IOC.";

reference "3GPP TS 28.541";

uses top3gpp:Top_Grp;

leaf cesSwitch {

description "This attribute determines whether the Cross Domain-Centralized SON energy saving function is enabled or disabled.";

type boolean;

}

leaf energySavingState {

description "Specifies the status regarding the energy saving in the cell. If the value of energySavingControl is toBeEnergySaving, then it shall be tried to achieve the value isEnergySaving for the energySavingState. If the value of energySavingControl is toBeNotEnergySaving, then it shall be tried to achieve the value isNotEnergySaving for the energySavingState. ";

type enumeration{

enum isNotEnergySaving;

enum isEnergySaving;

}

}

leaf energySavingControl {

description "This attribute allows the Cross Domain-Centralized SON energy saving function to initiate energy saving activation or deactivation.";

type enumeration{

enum toBeEnergySaving;

enum toBeNotEnergySaving;

}

}

}

augment "/me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction/nrcellcu3gpp:NRCellCU" {

if-feature nrcellcu3gpp:CESManagementFunction;

uses CESManagementFunctionGrp;

}

augment "/me3gpp:ManagedElement" {

if-feature me3gpp:CESManagementFunction;

uses CESManagementFunctionGrp;

}

augment "/subnet3gpp:SubNetwork" {

if-feature subnet3gpp:CESManagementFunction;

uses CESManagementFunctionGrp;

}

}

# E.6	Void

# E.7	Mount information

At the mountpoint "children-of-SubNetwork" in the YANG module _3gpp-common-subnetwork, the following YANG modules may be mounted if the class ManagedElement and the underlying hierarchy is contained under a SubNetwork.

See [45] that describes the mechanism that adds the schema trees defined by a set of YANG modules onto a mount point defined in the schema tree in another YANG module.

_3gpp-common-ep-rp.yang

_3gpp-common-managed-element.yang

_3gpp-common-managed-function.yang

_3gpp-common-measurements.yang

_3gpp-common-subnetwork.yang

_3gpp-common-top.yang

_3gpp-common-yang-extensions.yang

_3gpp-common-yang-types.yang

_3gpp-nr-nrm-bwp.yang

_3gpp-nr-nrm-ep.yang

_3gpp-nr-nrm-eutrancellrelation.yang

_3gpp-nr-nrm-gnbcucpfunction.yang

_3gpp-nr-nrm-gnbcuupfunction.yang

_3gpp-nr-nrm-gnbdufunction.yang

_3gpp-nr-nrm-nrcellcu.yang

_3gpp-nr-nrm-nrcelldu.yang

_3gpp-nr-nrm-nrcellrelation.yang

_3gpp-nr-nrm-nrfreqrelation.yang

_3gpp-nr-nrm-nrfrequency.yang

_3gpp-nr-nrm-nrnetwork.yang

_3gpp-nr-nrm-nrsectorcarrier.yang

_3gpp-nr-nrm-beam.yang

_3gpp-nr-nrm-commonbeamformingfunction.yang

_3gpp-nr-nrm-rrmpolicy.yang

ietf-inet-types.yang

ietf-yang-types.yang

If the above files are mounted the yang files described in clause H.7 shall also be mounted.

Annex F (normative):
Void

Annex G (normative):
OpenAPI definition of the 5GC NRM

# G.1	General

This annex contains the OpenAPI definition of the NR NRM in YAML format.

The Information Service (IS) of the NR NRM is defined in clause 4.

Mapping rules to produce the OpenAPI definition based on the IS are defined in 3GPP TS 32.160 [14].

# G.2	Void

# G.3	Void

# G.4	Solution Set (SS) definitions

## G.4.1	Void

## G.4.2	Void

## G.4.3	OpenAPI document "5gcNrm.yaml"

openapi: 3.0.1

info:

title: 3GPP 5GC NRM

version: 17.2.0

description: >-

OAS 3.0.1 specification of the 5GC NRM

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.541 V17.2.0; 5G NRM, 5GC NRM

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.541/

paths: {}

components:

schemas:

#-------- Definition of types-----------------------------------------------------

AmfIdentifier:

type: object

description: 'AmfIdentifier comprise of amfRegionId, amfSetId and amfPointer'

properties:

amfRegionId:

$ref: '#/components/schemas/AmfRegionId'

amfSetId:

$ref: '#/components/schemas/AmfSetId'

amfPointer:

$ref: '#/components/schemas/AmfPointer'

AmfRegionId:

type: integer

description: AmfRegionId is defined in TS 23.003

maximum: 255

AmfSetId:

type: string

description: AmfSetId is defined in TS 23.003

maximum: 1023

AmfPointer:

type: integer

description: AmfPointer is defined in TS 23.003

maximum: 63

IpEndPoint:

type: object

properties:

ipv4Address:

$ref: 'genericNrm.yaml#/components/schemas/Ipv4Addr'

ipv6Address:

$ref: 'genericNrm.yaml#/components/schemas/Ipv6Addr'

ipv6Prefix:

$ref: 'genericNrm.yaml#/components/schemas/Ipv6Prefix'

transport:

$ref: 'genericNrm.yaml#/components/schemas/TransportProtocol'

port:

type: integer

NFProfileList:

type: array

description: List of NF profile

items:

$ref: '#/components/schemas/NFProfile'

NFProfile:

type: object

description: 'NF profile stored in NRF, defined in TS 29.510'

properties:

nFInstanceId:

type: string

description: uuid of NF instance

nFType:

$ref: 'genericNrm.yaml#/components/schemas/NFType'

nFStatus:

$ref: '#/components/schemas/NFStatus'

plmn:

$ref: 'nrNrm.yaml#/components/schemas/PlmnId'

sNssais:

$ref: 'nrNrm.yaml#/components/schemas/Snssai'

fqdn:

$ref: 'genericNrm.yaml#/components/schemas/Fqdn'

interPlmnFqdn:

$ref: 'genericNrm.yaml#/components/schemas/Fqdn'

nfServices:

type: array

items:

$ref: '#/components/schemas/NFService'

NFService:

type: object

description: NF Service is defined in TS 29.510

properties:

serviceInstanceId:

type: string

serviceName:

type: string

version:

type: string

schema:

type: string

fqdn:

$ref: 'genericNrm.yaml#/components/schemas/Fqdn'

interPlmnFqdn:

$ref: 'genericNrm.yaml#/components/schemas/Fqdn'

ipEndPoints:

type: array

items:

$ref: '#/components/schemas/IpEndPoint'

apiPrfix:

type: string

allowedPlmns:

$ref: 'nrNrm.yaml#/components/schemas/PlmnId'

allowedNfTypes:

type: array

items:

$ref: 'genericNrm.yaml#/components/schemas/NFType'

allowedNssais:

type: array

items:

$ref: 'nrNrm.yaml#/components/schemas/Snssai'

NFStatus:

type: string

description: any of enumrated value

enum:

- REGISTERED

- SUSPENDED

CNSIIdList:

type: array

items:

$ref: '#/components/schemas/CNSIId'

CNSIId:

type: string

description: CNSI Id is defined in TS 29.531, only for Core Network

TACList:

type: array

items:

$ref: 'nrNrm.yaml#/components/schemas/NrTac'

WeightFactor:

type: integer

UdmInfo:

type: object

properties:

nFSrvGroupId:

type: string

AusfInfo:

type: object

properties:

nFSrvGroupId:

type: string

UpfInfo:

type: object

properties:

smfServingAreas:

type: string

AmfInfo:

type: object

properties:

priority:

type: integer

SupportedDataSetId:

type: string

description: any of enumrated value

enum:

- SUBSCRIPTION

- POLICY

- EXPOSURE

- APPLICATION

Udrinfo:

type: object

properties:

supportedDataSetIds:

type: array

items:

$ref: '#/components/schemas/SupportedDataSetId'

nFSrvGroupId:

type: string

NFInfo:

oneOf:

- $ref: '#/components/schemas/UdmInfo'

- $ref: '#/components/schemas/AusfInfo'

- $ref: '#/components/schemas/UpfInfo'

- $ref: '#/components/schemas/AmfInfo'

- $ref: '#/components/schemas/Udrinfo'

ManagedNFProfile:

type: object

properties:

nfInstanceID:

type: string

nfType:

$ref: 'genericNrm.yaml#/components/schemas/NFType'

authzInfo:

type: string

hostAddr:

$ref: 'genericNrm.yaml#/components/schemas/HostAddr'

locality:

type: string

nFInfo:

$ref: '#/components/schemas/NFInfo'

capacity:

type: integer

SEPPType:

type: string

description: any of enumrated value

enum:

- CSEPP

- PSEPP

SupportedFunc:

type: object

properties:

function:

type: string

policy:

type: string

SupportedFuncList:

type: array

items:

$ref: '#/components/schemas/SupportedFunc'

CommModelType:

type: string

description: any of enumrated value

enum:

- DIRECT_COMMUNICATION_WO_NRF

- DIRECT_COMMUNICATION_WITH_NRF

- INDIRECT_COMMUNICATION_WO_DEDICATED_DISCOVERY

- INDIRECT_COMMUNICATION_WITH_DEDICATED_DISCOVERY

CommModel:

type: object

properties:

groupId:

type: integer

commModelType:

$ref: '#/components/schemas/CommModelType'

targetNFServiceList:

$ref: 'genericNrm.yaml#/components/schemas/DnList'

commModelConfiguration:

type: string

CommModelList:

type: array

items:

$ref: '#/components/schemas/CommModel'

CapabilityList:

type: array

items:

type: string

FiveQiDscpMapping:

type: object

properties:

fiveQIValues:

type: array

items:

type: integer

dscp:

type: integer

PacketErrorRate:

type: object

properties:

scalar:

type: integer

exponent:

type: integer

FiveQICharacteristics:

type: object

properties:

fiveQIValue:

type: integer

resourceType:

type: string

enum:

- GBR

- NonGBR

priorityLevel:

type: integer

packetDelayBudget:

type: integer

packetErrorRate:

$ref: '#/components/schemas/PacketErrorRate'

averagingWindow:

type: integer

maximumDataBurstVolume:

type: integer

GtpUPathDelayThresholdsType:

type: object

properties:

n3AveragePacketDelayThreshold:

type: integer

n3MinPacketDelayThreshold:

type: integer

n3MaxPacketDelayThreshold:

type: integer

n9AveragePacketDelayThreshold:

type: integer

n9MinPacketDelayThreshold:

type: integer

n9MaxPacketDelayThreshold:

type: integer

QFPacketDelayThresholdsType:

type: object

properties:

thresholdDl:

type: integer

thresholdUl:

type: integer

thresholdRtt:

type: integer

QosData:

type: object

properties:

qosId:

type: string

fiveQIValue:

type: integer

maxbrUl:

$ref: 'https://forge.3gpp.org/rep/all/5G_APIs/raw/REL-16/TS29571_CommonData.yaml#/components/schemas/BitRateRm'

maxbrDl:

$ref: 'https://forge.3gpp.org/rep/all/5G_APIs/raw/REL-16/TS29571_CommonData.yaml#/components/schemas/BitRateRm'

gbrUl:

$ref: 'https://forge.3gpp.org/rep/all/5G_APIs/raw/REL-16/TS29571_CommonData.yaml#/components/schemas/BitRateRm'

gbrDl:

$ref: 'https://forge.3gpp.org/rep/all/5G_APIs/raw/REL-16/TS29571_CommonData.yaml#/components/schemas/BitRateRm'

arp:

$ref: 'https://forge.3gpp.org/rep/all/5G_APIs/raw/REL-16/TS29571_CommonData.yaml#/components/schemas/Arp'

qosNotificationControl:

type: boolean

reflectiveQos:

type: boolean

sharingKeyDl:

type: string

sharingKeyUl:

type: string

maxPacketLossRateDl:

$ref: 'https://forge.3gpp.org/rep/all/5G_APIs/raw/REL-16/TS29571_CommonData.yaml#/components/schemas/PacketLossRateRm'

maxPacketLossRateUl:

$ref: 'https://forge.3gpp.org/rep/all/5G_APIs/raw/REL-16/TS29571_CommonData.yaml#/components/schemas/PacketLossRateRm'

extMaxDataBurstVol:

$ref: 'https://forge.3gpp.org/rep/all/5G_APIs/raw/REL-16/TS29571_CommonData.yaml#/components/schemas/ExtMaxDataBurstVolRm'

QosDataList:

type: array

items:

$ref: '#/components/schemas/QosData'

SteeringMode:

type: object

properties:

steerModeValue:

$ref: 'https://forge.3gpp.org/rep/all/5G_APIs/raw/REL-16/TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/SteerModeValue'

active:

$ref: 'https://forge.3gpp.org/rep/all/5G_APIs/raw/REL-16/TS29571_CommonData.yaml#/components/schemas/AccessType'

standby:

$ref: 'https://forge.3gpp.org/rep/all/5G_APIs/raw/REL-16/TS29571_CommonData.yaml#/components/schemas/AccessTypeRm'

threeGLoad:

$ref: 'https://forge.3gpp.org/rep/all/5G_APIs/raw/REL-16/TS29571_CommonData.yaml#/components/schemas/Uinteger'

prioAcc:

$ref: 'https://forge.3gpp.org/rep/all/5G_APIs/raw/REL-16/TS29571_CommonData.yaml#/components/schemas/AccessType'

TrafficControlData:

type: object

properties:

tcId:

type: string

flowStatus:

$ref: 'https://forge.3gpp.org/rep/all/5G_APIs/raw/REL-16/TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/FlowStatus'

redirectInfo:

$ref: 'https://forge.3gpp.org/rep/all/5G_APIs/raw/REL-16/TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/RedirectInformation'

addRedirectInfo:

type: array

items:

$ref: 'https://forge.3gpp.org/rep/all/5G_APIs/raw/REL-16/TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/RedirectInformation'

minItems: 1

muteNotif:

type: boolean

trafficSteeringPolIdDl:

type: string

nullable: true

trafficSteeringPolIdUl:

type: string

nullable: true

routeToLocs:

type: array

items:

$ref: 'https://forge.3gpp.org/rep/all/5G_APIs/raw/REL-16/TS29571_CommonData.yaml#/components/schemas/RouteToLocation'

traffCorreInd:

type: boolean

upPathChgEvent:

$ref: 'https://forge.3gpp.org/rep/all/5G_APIs/raw/REL-16/TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/UpPathChgEvent'

steerFun:

$ref: 'https://forge.3gpp.org/rep/all/5G_APIs/raw/REL-16/TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/SteeringFunctionality'

steerModeDl:

$ref: '#/components/schemas/SteeringMode'

steerModeUl:

$ref: '#/components/schemas/SteeringMode'

mulAccCtrl:

$ref: 'https://forge.3gpp.org/rep/all/5G_APIs/raw/REL-16/TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/MulticastAccessControl'

TrafficControlDataList:

type: array

items:

$ref: '#/components/schemas/TrafficControlData'

PccRule:

type: object

properties:

pccRuleId:

type: string

description: Univocally identifies the PCC rule within a PDU session.

flowInfoList:

type: array

items:

$ref: 'https://forge.3gpp.org/rep/all/5G_APIs/raw/REL-16/TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/FlowInformation'

applicationId:

type: string

appDescriptor:

$ref: 'https://forge.3gpp.org/rep/all/5G_APIs/raw/REL-16/TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/ApplicationDescriptor'

contentVersion:

$ref: 'https://forge.3gpp.org/rep/all/5G_APIs/raw/REL-16/TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/ContentVersion'

precedence:

$ref: 'https://forge.3gpp.org/rep/all/5G_APIs/raw/REL-16/TS29571_CommonData.yaml#/components/schemas/Uinteger'

afSigProtocol:

$ref: 'https://forge.3gpp.org/rep/all/5G_APIs/raw/REL-16/TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/AfSigProtocol'

isAppRelocatable:

type: boolean

isUeAddrPreserved:

type: boolean

qosData:

type: array

items:

$ref: '#/components/schemas/QosDataList'

altQosParams:

type: array

items:

$ref: '#/components/schemas/QosDataList'

trafficControlData:

type: array

items:

$ref: '#/components/schemas/TrafficControlDataList'

conditionData:

$ref: 'https://forge.3gpp.org/rep/all/5G_APIs/raw/REL-16/TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/ConditionData'

tscaiInputDl:

$ref: 'https://forge.3gpp.org/rep/all/5G_APIs/raw/REL-16/TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/TscaiInputContainer'

tscaiInputUl:

$ref: 'https://forge.3gpp.org/rep/all/5G_APIs/raw/REL-16/TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/TscaiInputContainer'

#-------- Definition of concrete IOCs --------------------------------------------

SubNetwork-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/SubNetwork-Attr'

- $ref: 'genericNrm.yaml#/components/schemas/SubNetwork-ncO'

- type: object

properties:

SubNetwork:

$ref: '#/components/schemas/SubNetwork-Multiple'

ManagedElement:

$ref: '#/components/schemas/ManagedElement-Multiple'

ExternalAmfFunction:

$ref: '#/components/schemas/ExternalAmfFunction-Multiple'

ExternalNrfFunction:

$ref: '#/components/schemas/ExternalNrfFunction-Multiple'

ExternalNssfFunction:

$ref: '#/components/schemas/ExternalNssfFunction-Multiple'

AmfSet:

$ref: '#/components/schemas/AmfSet-Multiple'

AmfRegion:

$ref: '#/components/schemas/AmfRegion-Multiple'

Configurable5QISet:

$ref: '#/components/schemas/Configurable5QISet-Multiple'

Dynamic5QISet:

$ref: '#/components/schemas/Dynamic5QISet-Multiple'

ManagedElement-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedElement-Attr'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedElement-ncO'

- type: object

properties:

AmfFunction:

$ref: '#/components/schemas/AmfFunction-Multiple'

SmfFunction:

$ref: '#/components/schemas/SmfFunction-Multiple'

UpfFunction:

$ref: '#/components/schemas/UpfFunction-Multiple'

N3iwfFunction:

$ref: '#/components/schemas/N3iwfFunction-Multiple'

PcfFunction:

$ref: '#/components/schemas/PcfFunction-Multiple'

AusfFunction:

$ref: '#/components/schemas/AusfFunction-Multiple'

UdmFunction:

$ref: '#/components/schemas/UdmFunction-Multiple'

UdrFunction:

$ref: '#/components/schemas/UdrFunction-Multiple'

UdsfFunction:

$ref: '#/components/schemas/UdsfFunction-Multiple'

NrfFunction:

$ref: '#/components/schemas/NrfFunction-Multiple'

NssfFunction:

$ref: '#/components/schemas/NssfFunction-Multiple'

SmsfFunction:

$ref: '#/components/schemas/SmsfFunction-Multiple'

LmfFunction:

$ref: '#/components/schemas/LmfFunction-Multiple'

NgeirFunction:

$ref: '#/components/schemas/NgeirFunction-Multiple'

SeppFunction:

$ref: '#/components/schemas/SeppFunction-Multiple'

NwdafFunction:

$ref: '#/components/schemas/NwdafFunction-Multiple'

ScpFunction:

$ref: '#/components/schemas/ScpFunction-Multiple'

NefFunction:

$ref: '#/components/schemas/NefFunction-Multiple'

Configurable5QISet:

$ref: '#/components/schemas/Configurable5QISet-Multiple'

Dynamic5QISet:

$ref: '#/components/schemas/Dynamic5QISet-Multiple'

AmfFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

plmnIdList:

$ref: 'nrNrm.yaml#/components/schemas/PlmnIdList'

amfIdentifier:

$ref: '#/components/schemas/AmfIdentifier'

sBIFqdn:

type: string

weightFactor:

$ref: '#/components/schemas/WeightFactor'

snssaiList:

$ref: 'nrNrm.yaml#/components/schemas/SnssaiList'

amfSet:

$ref: 'genericNrm.yaml#/components/schemas/Dn'

managedNFProfile:

$ref: '#/components/schemas/ManagedNFProfile'

commModelList:

$ref: '#/components/schemas/CommModelList'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

EP_N2:

$ref: '#/components/schemas/EP_N2-Multiple'

EP_N8:

$ref: '#/components/schemas/EP_N8-Multiple'

EP_N11:

$ref: '#/components/schemas/EP_N11-Multiple'

EP_N12:

$ref: '#/components/schemas/EP_N12-Multiple'

EP_N14:

$ref: '#/components/schemas/EP_N14-Multiple'

EP_N15:

$ref: '#/components/schemas/EP_N15-Multiple'

EP_N17:

$ref: '#/components/schemas/EP_N17-Multiple'

EP_N20:

$ref: '#/components/schemas/EP_N20-Multiple'

EP_N22:

$ref: '#/components/schemas/EP_N22-Multiple'

EP_N26:

$ref: '#/components/schemas/EP_N26-Multiple'

EP_NLS:

$ref: '#/components/schemas/EP_NLS-Multiple'

EP_NLG:

$ref: '#/components/schemas/EP_NLG-Multiple'

AmfSet-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

plmnIdList:

$ref: 'nrNrm.yaml#/components/schemas/PlmnIdList'

nRTACList:

$ref: '#/components/schemas/TACList'

amfSetId:

$ref: '#/components/schemas/AmfSetId'

snssaiList:

$ref: 'nrNrm.yaml#/components/schemas/SnssaiList'

AmfRegion-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

plmnIdList:

$ref: 'nrNrm.yaml#/components/schemas/PlmnIdList'

nRTACList:

$ref: '#/components/schemas/TACList'

amfRegionId:

$ref: '#/components/schemas/AmfRegionId'

snssaiList:

$ref: 'nrNrm.yaml#/components/schemas/SnssaiList'

SmfFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

plmnIdList:

$ref: 'nrNrm.yaml#/components/schemas/PlmnIdList'

nRTACList:

$ref: '#/components/schemas/TACList'

sBIFqdn:

type: string

snssaiList:

$ref: 'nrNrm.yaml#/components/schemas/SnssaiList'

managedNFProfile:

$ref: '#/components/schemas/ManagedNFProfile'

commModelList:

$ref: '#/components/schemas/CommModelList'

configurable5QISetRef:

$ref: 'genericNrm.yaml#/components/schemas/Dn'

dynamic5QISetRef:

$ref: 'genericNrm.yaml#/components/schemas/Dn'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

EP_N4:

$ref: '#/components/schemas/EP_N4-Multiple'

EP_N7:

$ref: '#/components/schemas/EP_N7-Multiple'

EP_N10:

$ref: '#/components/schemas/EP_N10-Multiple'

EP_N11:

$ref: '#/components/schemas/EP_N11-Multiple'

EP_N16:

$ref: '#/components/schemas/EP_N16-Multiple'

EP_S5C:

$ref: '#/components/schemas/EP_S5C-Multiple'

FiveQiDscpMappingSet:

$ref: '#/components/schemas/FiveQiDscpMappingSet-Single'

GtpUPathQoSMonitoringControl:

$ref: '#/components/schemas/GtpUPathQoSMonitoringControl-Single'

QFQoSMonitoringControl:

$ref: '#/components/schemas/QFQoSMonitoringControl-Single'

PredefinedPccRuleSet:

$ref: '#/components/schemas/PredefinedPccRuleSet-Single'

UpfFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

plmnIdList:

$ref: 'nrNrm.yaml#/components/schemas/PlmnIdList'

nRTACList:

$ref: '#/components/schemas/TACList'

snssaiList:

$ref: 'nrNrm.yaml#/components/schemas/SnssaiList'

managedNFProfile:

$ref: '#/components/schemas/ManagedNFProfile'

commModelList:

$ref: '#/components/schemas/CommModelList'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

EP_N3:

$ref: '#/components/schemas/EP_N3-Multiple'

EP_N4:

$ref: '#/components/schemas/EP_N4-Multiple'

EP_N6:

$ref: '#/components/schemas/EP_N6-Multiple'

EP_N9:

$ref: '#/components/schemas/EP_N9-Multiple'

EP_S5U:

$ref: '#/components/schemas/EP_S5U-Multiple'

N3iwfFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

plmnIdList:

$ref: 'nrNrm.yaml#/components/schemas/PlmnIdList'

commModelList:

$ref: '#/components/schemas/CommModelList'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

EP_N3:

$ref: '#/components/schemas/EP_N3-Multiple'

EP_N4:

$ref: '#/components/schemas/EP_N4-Multiple'

PcfFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

plmnIdList:

$ref: 'nrNrm.yaml#/components/schemas/PlmnIdList'

sBIFqdn:

type: string

snssaiList:

$ref: 'nrNrm.yaml#/components/schemas/SnssaiList'

managedNFProfile:

$ref: '#/components/schemas/ManagedNFProfile'

commModelList:

$ref: '#/components/schemas/CommModelList'

configurable5QISetRef:

$ref: 'genericNrm.yaml#/components/schemas/Dn'

dynamic5QISetRef:

$ref: 'genericNrm.yaml#/components/schemas/Dn'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

EP_N5:

$ref: '#/components/schemas/EP_N5-Multiple'

EP_N7:

$ref: '#/components/schemas/EP_N7-Multiple'

EP_N15:

$ref: '#/components/schemas/EP_N15-Multiple'

EP_N16:

$ref: '#/components/schemas/EP_N16-Multiple'

EP_Rx:

$ref: '#/components/schemas/EP_Rx-Multiple'

PredefinedPccRuleSet:

$ref: '#/components/schemas/PredefinedPccRuleSet-Single'

AusfFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

plmnIdList:

$ref: 'nrNrm.yaml#/components/schemas/PlmnIdList'

sBIFqdn:

type: string

snssaiList:

$ref: 'nrNrm.yaml#/components/schemas/SnssaiList'

managedNFProfile:

$ref: '#/components/schemas/ManagedNFProfile'

commModelList:

$ref: '#/components/schemas/CommModelList'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

EP_N12:

$ref: '#/components/schemas/EP_N12-Multiple'

EP_N13:

$ref: '#/components/schemas/EP_N13-Multiple'

UdmFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

plmnIdList:

$ref: 'nrNrm.yaml#/components/schemas/PlmnIdList'

sBIFqdn:

type: string

snssaiList:

$ref: 'nrNrm.yaml#/components/schemas/SnssaiList'

managedNFProfile:

$ref: '#/components/schemas/ManagedNFProfile'

commModelList:

$ref: '#/components/schemas/CommModelList'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

EP_N8:

$ref: '#/components/schemas/EP_N8-Multiple'

EP_N10:

$ref: '#/components/schemas/EP_N10-Multiple'

EP_N13:

$ref: '#/components/schemas/EP_N13-Multiple'

UdrFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

plmnIdList:

$ref: 'nrNrm.yaml#/components/schemas/PlmnIdList'

sBIFqdn:

type: string

snssaiList:

$ref: 'nrNrm.yaml#/components/schemas/SnssaiList'

managedNFProfile:

$ref: '#/components/schemas/ManagedNFProfile'

UdsfFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

plmnIdList:

$ref: 'nrNrm.yaml#/components/schemas/PlmnIdList'

sBIFqdn:

type: string

snssaiList:

$ref: 'nrNrm.yaml#/components/schemas/SnssaiList'

managedNFProfile:

$ref: '#/components/schemas/ManagedNFProfile'

NrfFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

plmnIdList:

$ref: 'nrNrm.yaml#/components/schemas/PlmnIdList'

sBIFqdn:

type: string

cNSIIdList:

$ref: '#/components/schemas/CNSIIdList'

nFProfileList:

$ref: '#/components/schemas/NFProfileList'

snssaiList:

$ref: 'nrNrm.yaml#/components/schemas/SnssaiList'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

EP_N27:

$ref: '#/components/schemas/EP_N27-Multiple'

NssfFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

plmnIdList:

$ref: 'nrNrm.yaml#/components/schemas/PlmnIdList'

sBIFqdn:

type: string

cNSIIdList:

$ref: '#/components/schemas/CNSIIdList'

nFProfileList:

$ref: '#/components/schemas/NFProfileList'

snssaiList:

$ref: 'nrNrm.yaml#/components/schemas/SnssaiList'

commModelList:

$ref: '#/components/schemas/CommModelList'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

EP_N22:

$ref: '#/components/schemas/EP_N22-Multiple'

EP_N31:

$ref: '#/components/schemas/EP_N31-Multiple'

SmsfFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

plmnIdList:

$ref: 'nrNrm.yaml#/components/schemas/PlmnIdList'

sBIFqdn:

type: string

managedNFProfile:

$ref: '#/components/schemas/ManagedNFProfile'

commModelList:

$ref: '#/components/schemas/CommModelList'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

EP_N20:

$ref: '#/components/schemas/EP_N20-Multiple'

EP_N21:

$ref: '#/components/schemas/EP_N21-Multiple'

EP_MAP_SMSC:

$ref: '#/components/schemas/EP_MAP_SMSC-Multiple'

LmfFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

plmnIdList:

$ref: 'nrNrm.yaml#/components/schemas/PlmnIdList'

managedNFProfile:

$ref: '#/components/schemas/ManagedNFProfile'

commModelList:

$ref: '#/components/schemas/CommModelList'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

EP_NLS:

$ref: '#/components/schemas/EP_NLS-Multiple'

NgeirFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

plmnIdList:

$ref: 'nrNrm.yaml#/components/schemas/PlmnIdList'

sBIFqdn:

type: string

snssaiList:

$ref: 'nrNrm.yaml#/components/schemas/SnssaiList'

managedNFProfile:

$ref: '#/components/schemas/ManagedNFProfile'

commModelList:

$ref: '#/components/schemas/CommModelList'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

EP_N17:

$ref: '#/components/schemas/EP_N17-Multiple'

SeppFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

plmnId:

$ref: 'nrNrm.yaml#/components/schemas/PlmnId'

sEPPType:

$ref: '#/components/schemas/SEPPType'

sEPPId:

type: integer

fqdn:

$ref: 'genericNrm.yaml#/components/schemas/Fqdn'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

EP_N32:

$ref: '#/components/schemas/EP_N32-Multiple'

NwdafFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

plmnIdList:

$ref: 'nrNrm.yaml#/components/schemas/PlmnIdList'

sBIFqdn:

type: string

snssaiList:

$ref: 'nrNrm.yaml#/components/schemas/SnssaiList'

managedNFProfile:

$ref: '#/components/schemas/ManagedNFProfile'

commModelList:

$ref: '#/components/schemas/CommModelList'

ScpFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

supportedFuncList:

$ref: '#/components/schemas/SupportedFuncList'

address:

$ref: 'genericNrm.yaml#/components/schemas/HostAddr'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

NefFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

sBIFqdn:

type: string

snssaiList:

$ref: 'nrNrm.yaml#/components/schemas/SnssaiList'

managedNFProfile:

$ref: '#/components/schemas/ManagedNFProfile'

capabilityList:

$ref: '#/components/schemas/CapabilityList'

isINEF:

type: boolean

isCAPIFSup:

type: boolean

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

ExternalAmfFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

plmnIdList:

$ref: 'nrNrm.yaml#/components/schemas/PlmnIdList'

amfIdentifier:

$ref: '#/components/schemas/AmfIdentifier'

ExternalNrfFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

plmnIdList:

$ref: 'nrNrm.yaml#/components/schemas/PlmnIdList'

ExternalNssfFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

plmnIdList:

$ref: 'nrNrm.yaml#/components/schemas/PlmnIdList'

ExternalSeppFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

plmnId:

$ref: 'nrNrm.yaml#/components/schemas/PlmnId'

sEPPId:

type: integer

fqdn:

$ref: 'genericNrm.yaml#/components/schemas/Fqdn'

EP_N2-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_N3-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

epTransportRefs:

$ref: 'genericNrm.yaml#/components/schemas/DnList'

EP_N4-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_N5-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_N6-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_N7-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_N8-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_N9-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_N10-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_N11-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_N12-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_N13-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_N14-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_N15-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_N16-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_N17-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_N20-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_N21-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_N22-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_N26-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_N27-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_N31-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_N32-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

remotePlmnId:

$ref: 'nrNrm.yaml#/components/schemas/PlmnId'

remoteSeppAddress:

$ref: 'genericNrm.yaml#/components/schemas/HostAddr'

remoteSeppId:

type: integer

n32cParas:

type: string

n32fPolicy:

type: string

withIPX:

type: boolean

EP_S5C-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_S5U-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_Rx-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_MAP_SMSC-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_NLS-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

EP_NLG-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/EP_RP-Attr'

- type: object

properties:

localAddress:

$ref: 'nrNrm.yaml#/components/schemas/LocalAddress'

remoteAddress:

$ref: 'nrNrm.yaml#/components/schemas/RemoteAddress'

FiveQiDscpMappingSet-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

FiveQiDscpMappingList:

type: array

items:

$ref: '#/components/schemas/FiveQiDscpMapping'

Configurable5QISet-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

configurable5QIs:

type: array

items:

$ref: '#/components/schemas/FiveQICharacteristics'

Dynamic5QISet-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

dynamic5QIs:

type: array

items:

$ref: '#/components/schemas/FiveQICharacteristics'

GtpUPathQoSMonitoringControl-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

gtpUPathQoSMonitoringState:

type: string

enum:

- ENABLED

- DISABLED

gtpUPathMonitoredSNSSAIs:

type: array

items:

$ref: 'nrNrm.yaml#/components/schemas/Snssai'

monitoredDSCPs:

type: array

items:

type: integer

minimum: 0

maximum: 255

isEventTriggeredGtpUPathMonitoringSupported:

type: boolean

isPeriodicGtpUMonitoringSupported:

type: boolean

isImmediateGtpUMonitoringSupported:

type: boolean

gtpUPathDelayThresholds:

$ref: '#/components/schemas/GtpUPathDelayThresholdsType'

gtpUPathMinimumWaitTime:

type: integer

gtpUPathMeasurementPeriod:

type: integer

QFQoSMonitoringControl-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

qFQoSMonitoringState:

type: string

enum:

- ENABLED

- DISABLED

qFMonitoredSNSSAIs:

type: array

items:

$ref: 'nrNrm.yaml#/components/schemas/Snssai'

qFMonitored5QIs:

type: array

items:

type: integer

minimum: 0

maximum: 255

isEventTriggeredQFMonitoringSupported:

type: boolean

isPeriodicQFMonitoringSupported:

type: boolean

isSessionReleasedQFMonitoringSupported:

type: boolean

qFPacketDelayThresholds:

$ref: '#/components/schemas/QFPacketDelayThresholdsType'

qFMinimumWaitTime:

type: integer

qFMeasurementPeriod:

type: integer

PredefinedPccRuleSet-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

predefinedPccRules:

type: array

items:

$ref: '#/components/schemas/PccRule'

#-------- Definition of JSON arrays for name-contained IOCs ----------------------

SubNetwork-Multiple:

type: array

items:

$ref: '#/components/schemas/SubNetwork-Single'

ManagedElement-Multiple:

type: array

items:

$ref: '#/components/schemas/ManagedElement-Single'

AmfFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/AmfFunction-Single'

SmfFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/SmfFunction-Single'

UpfFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/UpfFunction-Single'

N3iwfFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/N3iwfFunction-Single'

PcfFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/PcfFunction-Single'

AusfFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/AusfFunction-Single'

UdmFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/UdmFunction-Single'

UdrFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/UdrFunction-Single'

UdsfFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/UdsfFunction-Single'

NrfFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/NrfFunction-Single'

NssfFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/NssfFunction-Single'

SmsfFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/SmsfFunction-Single'

LmfFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/LmfFunction-Single'

NgeirFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/NgeirFunction-Single'

SeppFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/SeppFunction-Single'

NwdafFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/NwdafFunction-Single'

ScpFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/ScpFunction-Single'

NefFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/NefFunction-Single'

ExternalAmfFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/ExternalAmfFunction-Single'

ExternalNrfFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/ExternalNrfFunction-Single'

ExternalNssfFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/ExternalNssfFunction-Single'

ExternalSeppFunction-Nultiple:

type: array

items:

$ref: '#/components/schemas/ExternalSeppFunction-Single'

AmfSet-Multiple:

type: array

items:

$ref: '#/components/schemas/AmfSet-Single'

AmfRegion-Multiple:

type: array

items:

$ref: '#/components/schemas/AmfRegion-Single'

EP_N2-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N2-Single'

EP_N3-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N3-Single'

EP_N4-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N4-Single'

EP_N5-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N5-Single'

EP_N6-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N6-Single'

EP_N7-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N7-Single'

EP_N8-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N8-Single'

EP_N9-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N9-Single'

EP_N10-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N10-Single'

EP_N11-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N11-Single'

EP_N12-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N12-Single'

EP_N13-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N13-Single'

EP_N14-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N14-Single'

EP_N15-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N15-Single'

EP_N16-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N16-Single'

EP_N17-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N17-Single'

EP_N20-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N20-Single'

EP_N21-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N21-Single'

EP_N22-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N22-Single'

EP_N26-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N26-Single'

EP_N27-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N27-Single'

EP_N31-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N31-Single'

EP_N32-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_N32-Single'

EP_S5C-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_S5C-Single'

EP_S5U-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_S5U-Single'

EP_Rx-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_Rx-Single'

EP_MAP_SMSC-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_MAP_SMSC-Single'

EP_NLS-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_NLS-Single'

EP_NLG-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_NLG-Single'

Configurable5QISet-Multiple:

type: array

items:

$ref: '#/components/schemas/Configurable5QISet-Single'

Dynamic5QISet-Multiple:

type: array

items:

$ref: '#/components/schemas/Dynamic5QISet-Single'

#------------ Definitions in TS 28.541 for TS 28.532 -----------------------------

resources-5gcNrm:

oneOf:

- $ref: '#/components/schemas/SubNetwork-Single'

- $ref: '#/components/schemas/ManagedElement-Single'

- $ref: '#/components/schemas/AmfFunction-Single'

- $ref: '#/components/schemas/SmfFunction-Single'

- $ref: '#/components/schemas/UpfFunction-Single'

- $ref: '#/components/schemas/N3iwfFunction-Single'

- $ref: '#/components/schemas/PcfFunction-Single'

- $ref: '#/components/schemas/AusfFunction-Single'

- $ref: '#/components/schemas/UdmFunction-Single'

- $ref: '#/components/schemas/UdrFunction-Single'

- $ref: '#/components/schemas/UdsfFunction-Single'

- $ref: '#/components/schemas/NrfFunction-Single'

- $ref: '#/components/schemas/NssfFunction-Single'

- $ref: '#/components/schemas/SmsfFunction-Single'

- $ref: '#/components/schemas/LmfFunction-Single'

- $ref: '#/components/schemas/NgeirFunction-Single'

- $ref: '#/components/schemas/SeppFunction-Single'

- $ref: '#/components/schemas/NwdafFunction-Single'

- $ref: '#/components/schemas/ScpFunction-Single'

- $ref: '#/components/schemas/NefFunction-Single'

- $ref: '#/components/schemas/ExternalAmfFunction-Single'

- $ref: '#/components/schemas/ExternalNrfFunction-Single'

- $ref: '#/components/schemas/ExternalNssfFunction-Single'

- $ref: '#/components/schemas/ExternalSeppFunction-Single'

- $ref: '#/components/schemas/AmfSet-Single'

- $ref: '#/components/schemas/AmfRegion-Single'

- $ref: '#/components/schemas/QFQoSMonitoringControl-Single'

- $ref: '#/components/schemas/GtpUPathQoSMonitoringControl-Single'

- $ref: '#/components/schemas/EP_N2-Single'

- $ref: '#/components/schemas/EP_N3-Single'

- $ref: '#/components/schemas/EP_N4-Single'

- $ref: '#/components/schemas/EP_N5-Single'

- $ref: '#/components/schemas/EP_N6-Single'

- $ref: '#/components/schemas/EP_N7-Single'

- $ref: '#/components/schemas/EP_N8-Single'

- $ref: '#/components/schemas/EP_N9-Single'

- $ref: '#/components/schemas/EP_N10-Single'

- $ref: '#/components/schemas/EP_N11-Single'

- $ref: '#/components/schemas/EP_N12-Single'

- $ref: '#/components/schemas/EP_N13-Single'

- $ref: '#/components/schemas/EP_N14-Single'

- $ref: '#/components/schemas/EP_N15-Single'

- $ref: '#/components/schemas/EP_N16-Single'

- $ref: '#/components/schemas/EP_N17-Single'

- $ref: '#/components/schemas/EP_N20-Single'

- $ref: '#/components/schemas/EP_N21-Single'

- $ref: '#/components/schemas/EP_N22-Single'

- $ref: '#/components/schemas/EP_N26-Single'

- $ref: '#/components/schemas/EP_N27-Single'

- $ref: '#/components/schemas/EP_N31-Single'

- $ref: '#/components/schemas/EP_N31-Single'

- $ref: '#/components/schemas/EP_S5C-Single'

- $ref: '#/components/schemas/EP_S5U-Single'

- $ref: '#/components/schemas/EP_Rx-Single'

- $ref: '#/components/schemas/EP_MAP_SMSC-Single'

- $ref: '#/components/schemas/EP_NLS-Single'

- $ref: '#/components/schemas/EP_NLG-Single'

- $ref: '#/components/schemas/Configurable5QISet-Single'

- $ref: '#/components/schemas/FiveQiDscpMappingSet-Single'

- $ref: '#/components/schemas/PredefinedPccRuleSet-Single'

- $ref: '#/components/schemas/Dynamic5QISet-Single'Annex H (normative):
YANG definitions for 5GC

# H.1	General

This annex contains the YANG definitions for the 5GC NRM, in accordance with 5GC information model definitions specified in clause 4.

# H.2	Void

# H.3	Void

# H.4	Void

# H.5	Modules

## H.5.1	module _3gpp-5gc-common-yang-types.yang

module _3gpp-5g-common-yang-types {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-5g-common-yang-types";

prefix "types5g3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "The model defines common types for 5G networks and

network slicing.";

reference "3GPP TS 28.541";

revision 2020-11-05 { reference CR-0412 ; }

revision 2019-10-20 { reference "Initial version."; }

grouping SNssai {

description

"Single Network Slice Selection Assistance Information(S-NSSAI)";

reference "3GPP TS 23.003";

leaf sd {

description "Slice Differentiator

If not needed, the value can be set to FFFFFF.";

type string{

length 6;

pattern '[a-fA-F0-9]*';

}

reference "3GPP TS 23.003";

}

leaf sst {

type uint8;

description "Slice/Service Type.

Values 0 to 127 belong to standardized SST range and are defined in

3GPP TS 23.501. Values 128 to 255 belong to operator-specific range.";

}

}

grouping PLMNInfo {

description "The PLMNInfo data type define a S-NSSAI member in a specific

PLMNId, and it have two attributes PLMNId and S-NSSAI (PLMNId, S-NSSAI).

The PLMNId represents a data type that is comprised of mcc

(mobile country code) and mnc (mobile network code), (See TS 23.003

subclause 2.2 and 12.1) and S-NSSAI represents an data type, that is

comprised of an SST (Slice/Service type) and an optional

SD (Slice Differentiator) field";

uses types3gpp:PLMNId;

uses SNssai;

}

typedef CommModelType {

reference "3GPP TS 23501";

type enumeration {

enum DIRECT_COMMUNICATION_WO_NRF {

value 0;

description "Directly communicate to other pre-configured NF service.";

}

enum DIRECT_COMMUNICATION_WITH_NRF {

value 1;

description "Directly communicate to other NF service discovered

by NRF.";

}

enum INDIRECT_COMMUNICATION_WO_DEDICATED_DISCOVERY {

value 2;

description "Communicate to pre-configured other NF service through

SCP as a proxy.";

}

enum INDIRECT_COMMUNICATION_WITH_DEDICATED_DISCOVERY {

value 3;

description "Communication to NF service discovered by NRF through SCP as a proxy.";

}

}

}

grouping CommModel {

leaf groupId {

type uint16;

}

leaf commModelType {

type CommModelType;

}

leaf-list targetNFServiceList {

type types3gpp:DistinguishedName;

}

leaf commModelConfiguration {

type string;

}

}

grouping SupportedFunc {

leaf function {

type string;

}

leaf policy {

type string;

}

}

}

## H.5.1a	module _3gpp-5gc-nrm-affunction@2019-10-28.yang

module _3gpp-5gc-nrm-affunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-affunction;

prefix af3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

description "This IOC is defined only to describe the IOCs representing

its interaction interface with 5GC (i.e. EP_Rx and EP_N5).

It has no attributes defined.";

reference "3GPP TS 28.541";

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-05-15 {

description "initial revision";

}

grouping AFFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

}

augment "/me3gpp:ManagedElement" {

list AFFunction {

description "5G Core AF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses AFFunctionGrp;

}

}

}

}

## H.5.2	module _3gpp-5gc-nrm-amffunction.yang

module _3gpp-5gc-nrm-amffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-amffunction;

prefix amf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "AMFFunction derived from basic ManagedFunction.";

revision 2020-11-05 { reference CR-0412 ; }

revision 2019-10-25 { reference "S5-194457 S5-193518"; }

revision 2019-05-31 { reference "Ericsson refactoring."; }

revision 2018-08-07 { reference "Initial revision"; }

grouping AMFFunctionGrp {

description "Represents the AMFFunction IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

min-elements 1;

description "A list of PLMN identifiers (Mobile Country Code and Mobile

Network Code).";

key "mcc mnc";

uses types3gpp:PLMNId;

}

container aMFIdentifier {

presence true;

description "An AMF identifier, comprising an AMF Region ID, an

AMF Set ID and an AMF Pointer.";

uses types3gpp:AmfIdentifier;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the

service-based interface.";

type inet:domain-name;

}

list sNSSAIList {

min-elements 1;

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

reference "3GPP TS 23.003";

key "sd sst";

uses types5g3gpp:SNssai;

}

list managedNFProfile {

key idx;

min-elements 1;

max-elements 1;

uses types3gpp:ManagedNFProfile;

}

list commModelList {

min-elements 1;

key "groupId";

description "Specifies a list of commModel. It can be used by NF and

NF services to interact with each other in 5G Core network ";

reference "3GPP TS 23.501";

uses types5g3gpp:CommModel;

}

}

augment "/me3gpp:ManagedElement" {

list AMFFunction {

description "5G Core AMF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses AMFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## H.5.3	module _3gpp-5gc-nrm-amfregion.yang

module _3gpp-5gc-nrm-amfregion {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-amfregion;

prefix amfr3gpp;

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the AMF Region which consists one or

multiple AMF Sets.";

revision 2020-11-05 { reference CR-0412 ; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-11 {

reference "Ericsson refactoring."; }

grouping AMFRegionGrp {

description "Represents the AMFRegion IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at

least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC)

and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf-list nRTACList {

description "List of Tracking Area Codes (legacy TAC or extended TAC)

where the represented management function is serving.";

reference "TS 38.413 clause 9.3.3.10";

min-elements 1;

config false;

type types3gpp:Tac;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//conditional support only if the network slicing feature is supported.

reference "3GPP TS 23.003";

key "sd sst";

uses types5g3gpp:SNssai;

}

leaf aMFRegionId {

description "Represents the AMF Region ID, which identifies the region.";

mandatory true;

type types3gpp:AmfRegionId;

}

leaf-list aMFSet {

description "The AMFSet that the AFMRegion is associated with.";

min-elements 1;

type instance-identifier;

}

}

augment "/subnet3gpp:SubNetwork" {

list AMFRegion {

description "5G Core AMFRegion IOC";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses AMFRegionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## H.5.4	module _3gpp-5gc-nrm-amfset.yang

module _3gpp-5gc-nrm-amfset {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-amfset;

prefix amfset3gpp;

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the AMF Set which consists of some AMFs

that serve a given area and Network Slice.";

revision 2020-11-05 { reference CR-0412 ; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-11 { reference "Ericsson refactoring."; }

grouping AMFSetGrp {

description "Represents the AMFSet IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at

least one (the primary PLMN Id). The PLMN Identifier is composed

of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf-list nRTACList {

description "List of Tracking Area Codes (legacy TAC or extended TAC)

where the represented management function is serving.";

reference "TS 38.413 clause 9.3.3.10";

min-elements 1;

config false;

type types3gpp:Tac;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//conditional support only if the network slicing feature is supported.

reference "3GPP TS 23.003";

key "sd sst";

uses types5g3gpp:SNssai;

}

leaf aMFRegion {

description "The AMFRegion that the AFMSet is associated with.";

type instance-identifier;

}

leaf-list aMFSetMemberList {

description "List of DNs of AMFFunction instances of the AMFSet.";

min-elements 1;

max-elements 1;

type types3gpp:DistinguishedName;

}

}

augment "/subnet3gpp:SubNetwork" {

list AMFSet {

description "5G Core AMFSet IOC";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses AMFSetGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## H.5.5	module _3gpp-5gc-nrm-ausffunction.yang

module _3gpp-5gc-nrm-ausffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-ausffunction;

prefix ausf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the AUSF function in 5GC. For more

information about the AUSF, see 3GPP TS 23.501.";

reference "3GPP TS 28.541";

revision 2020-11-05 { reference CR-0412 ; }

revision 2019-10-25 { reference "S5-194457 S5-193518"; }

revision 2019-05-22 {reference "initial revision"; }

grouping AUSFFuntionGrp {

description "Represents the AUSFFuntion IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at

least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and

a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the

service-based interface.";

type inet:domain-name;

}

list  sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//optional support

reference "3GPP TS 23.003";

key "sd sst";

uses types5g3gpp:SNssai;

}

list managedNFProfile {

key idx;

min-elements 1;

max-elements 1;

uses types3gpp:ManagedNFProfile;

}

list commModelList {

min-elements 1;

key "groupId";

description "Specifies a list of commModel. It can be used by NF and

NF services to interact with each other in 5G Core network ";

reference "3GPP TS 23.501";

uses types5g3gpp:CommModel;

}

}

augment "/me3gpp:ManagedElement" {

list AUSFFunction {

description "5G Core AUSF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses AUSFFuntionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## H.5.6	module _3gpp-5gc-nrm-dnfunction@2019-10-28.yang

module _3gpp-5gc-nrm-dnfunction {

yang-version 1.1;

namespace urn:3gpp:sa5_3gpp-5gc-nrm-dnfunction;

prefix dn3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

description "This IOC is defined only to describe the IOCs representing

Data Network (DN) interaction interface with 5GC (i.e. EP_N6).

It has no attributes defined.";

reference "3GPP TS 28.541";

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-05-15 {

description "initial revision";

}

grouping DNFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

}

augment "/me3gpp:ManagedElement" {

list DNFunction {

description "5G Core DN Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses DNFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## H.5.7	module _3gpp-5gc-nrm-ep@2019-11-18.yang

module _3gpp-5gc-nrm-ep {

yang-version 1.1;

namespace "urn:3gpp:tsg:sa5:nrm:_3gpp-5gc-nrm-ep";

prefix "cep3gpp";

import _3gpp-common-ep-rp { prefix eprp3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-5gc-nrm-affunction { prefix af3gpp; }

import _3gpp-5gc-nrm-amffunction { prefix amf3gpp; }

import _3gpp-5gc-nrm-ausffunction { prefix ausf3gpp; }

import _3gpp-5gc-nrm-dnfunction { prefix dn3gpp; }

import _3gpp-5gc-nrm-lmffunction { prefix lmf3gpp; }

import _3gpp-5gc-nrm-n3iwffunction { prefix n3iwf3gpp; }

import _3gpp-5gc-nrm-ngeirfunction { prefix ngeir3gpp; }

import _3gpp-5gc-nrm-nrffunction { prefix nrf3gpp; }

import _3gpp-5gc-nrm-nssffunction { prefix nssf3gpp; }

import _3gpp-5gc-nrm-pcffunction { prefix pcf3gpp; }

import _3gpp-5gc-nrm-seppfunction { prefix sepp3gpp; }

import _3gpp-5gc-nrm-smffunction { prefix smf3gpp; }

import _3gpp-5gc-nrm-smsffunction { prefix smsf3gpp; }

import _3gpp-5gc-nrm-udmfunction { prefix udm3gpp; }

import _3gpp-5gc-nrm-upffunction { prefix upf3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import ietf-inet-types { prefix inet; }

organization "3GPP SA5";

description "Defines the YANG mapping of the 5GC related endpoint

Information Object Classes (IOCs) that are part of the 5G Core

Network Resource Model.";

reference "3GPP TS 28.541";

revision 2019-11-18 {

description "Ericsson refactoring.";

}

revision 2018-07-31 {

description "Initial revision";

}

grouping EP_N2Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N3Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N4Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N5Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N6Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N7Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N8Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N9Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N10Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N11Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N12Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N13Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N14Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N15Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N16Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N17Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N20Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N21Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N22Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N26Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N27Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N31Grp {

uses eprp3gpp:EP_Common;

}

grouping EP_N32Grp {

uses eprp3gpp:EP_Common;

container remotePlmnId {

description "PLMN Identifiers of the remote sepp.

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

uses types3gpp:PLMNId;

}

leaf remoteSeppAddress {

description "The host address of the SEPP.";

type inet:host;

}

leaf remoteSeppId {

type uint16;

}

leaf n32cParas {

type string;

}

leaf n32fPolicy {

type string;

}

leaf withIPX {

type boolean;

}

}

grouping EP_S5CGrp {

uses eprp3gpp:EP_Common;

}

grouping EP_S5UGrp {

uses eprp3gpp:EP_Common;

}

grouping EP_RxGrp {

uses eprp3gpp:EP_Common;

}

grouping EP_MAP_SMSCGrp {

uses eprp3gpp:EP_Common;

}

grouping EP_NLSGrp {

uses eprp3gpp:EP_Common;

}

grouping EP_NLGGrp {

uses eprp3gpp:EP_Common;

}

grouping EP_SBI_IPXGrp {

uses eprp3gpp:EP_Common;

leaf-list sBIService {

min-elements 1;

config false;

type string;

}

}

augment "/me3gpp:ManagedElement/af3gpp:AFFunction" {

list EP_N6 {

description "Represents the EP_N6 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N6Grp;

}

}

list EP_Rx {

description "Represents the EP_Rx IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_RxGrp;

}

}

}

augment "/me3gpp:ManagedElement/amf3gpp:AMFFunction" {

list EP_N2 {

description "Represents the EP_N2 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N2Grp;

}

}

list EP_N8 {

description "Represents the EP_N8 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N8Grp;

}

}

list EP_N11 {

description "Represents the EP_N11 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N11Grp;

}

}

list EP_N12 {

description "Represents the EP_N12 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N12Grp;

}

}

list EP_N14 {

description "Represents the EP_N14 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N14Grp;

}

}

list EP_N15 {

description "Represents the EP_N15 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N15Grp;

}

}

list EP_N17 {

description "Represents the EP_N17 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N17Grp;

}

}

list EP_N20 {

description "Represents the EP_N20 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N20Grp;

}

}

list EP_N22 {

description "Represents the EP_N22 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N22Grp;

}

}

list EP_N26 {

description "Represents the EP_N26 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N26Grp;

}

}

list EP_NLS {

description "Represents the EP_NLS IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_NLSGrp;

}

}

list EP_NLG {

description "Represents the EP_NLG IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_NLGGrp;

}

}

}

augment "/me3gpp:ManagedElement/ausf3gpp:AUSFFunction" {

list EP_N12 {

description "Represents the EP_N12 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N12Grp;

}

}

list EP_N13 {

description "Represents the EP_N13 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N13Grp;

}

}

}

augment "/me3gpp:ManagedElement/dn3gpp:DNFunction" {

list EP_N6 {

description "Represents the EP_N6 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N6Grp;

}

}

}

augment "/me3gpp:ManagedElement/lmf3gpp:LMFFunction" {

list EP_NLS {

description "Represents the EP_NLS IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_NLSGrp;

}

}

}

augment "/me3gpp:ManagedElement/n3iwf3gpp:N3IWFFunction" {

list EP_N2 {

description "Represents the EP_N2 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N2Grp;

}

}

list EP_N3 {

description "Represents the EP_N3 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N3Grp;

}

}

}

augment "/me3gpp:ManagedElement/ngeir3gpp:NGEIRFunction" {

list EP_N17 {

description "Represents the EP_N17 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N17Grp;

}

}

}

augment "/me3gpp:ManagedElement/nrf3gpp:NRFFunction" {

list EP_N27 {

description "Represents the EP_N27 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N26Grp;

}

}

}

augment "/me3gpp:ManagedElement/nssf3gpp:NSSFFunction" {

list EP_N22 {

description "Represents the EP_N22 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N22Grp;

}

}

list EP_N31 {

description "Represents the EP_N31 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N31Grp;

}

}

}

augment "/me3gpp:ManagedElement/pcf3gpp:PCFFunction" {

list EP_N5 {

description "Represents the EP_N5 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N5Grp;

}

}

list EP_N7 {

description "Represents the EP_N7 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N7Grp;

}

}

list EP_N15 {

description "Represents the EP_N15 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N15Grp;

}

}

list EP_N16 {

description "Represents the EP_N16 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N16Grp;

}

}

list EP_Rx {

description "Represents the EP_Rx IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_RxGrp;

}

}

}

augment "/me3gpp:ManagedElement/sepp3gpp:SEPPFunction" {

list EP_N32 {

description "Represents the EP_N32 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N32Grp;

}

}

}

augment "/me3gpp:ManagedElement/smsf3gpp:SMSFFunction" {

list EP_N20 {

description "Represents the EP_20 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N20Grp;

}

}

list EP_N21 {

description "Represents the EP_N21 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N21Grp;

}

}

list EP_MAP_SMSC {

description "Represents the EP_MAP_SMSC IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_MAP_SMSCGrp;

}

}

}

augment "/me3gpp:ManagedElement/smf3gpp:SMFFunction" {

list EP_N4 {

description "Represents the EP_N4 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N4Grp;

}

}

list EP_N7 {

description "Represents the EP_N7 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N7Grp;

}

}

list EP_N10 {

description "Represents the EP_N10 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N10Grp;

}

}

list EP_N11 {

description "Represents the EP_N11 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N11Grp;

}

}

list EP_N16 {

description "Represents the EP_N16 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N16Grp;

}

}

list EP_S5C {

description "Represents the EP_S5C IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_S5CGrp;

}

}

}

augment "/me3gpp:ManagedElement/udm3gpp:UDMFunction" {

list EP_N8 {

description "Represents the EP_N8 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N8Grp;

}

}

list EP_N10 {

description "Represents the EP_N10 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N10Grp;

}

}

list EP_N13 {

description "Represents the EP_N13 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N13Grp;

}

}

}

augment "/me3gpp:ManagedElement/upf3gpp:UPFFunction" {

list EP_N4 {

description "Represents the EP_N4 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N4Grp;

}

}

list EP_N3 {

description "Represents the EP_N3 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N3Grp;

}

}

list EP_N9 {

description "Represents the EP_N9 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N9Grp;

}

}

list EP_S5U {

description "Represents the EP_S5U IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_S5UGrp;

}

}

list EP_EP_N6 {

description "Represents the EP_N6 IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EP_N6Grp;

}

}

}

}

## H.5.8	module _3gpp-5gc-nrm-externalnrffunction@2019-10-28.yang

module _3gpp-5gc-nrm-externalnrffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-externalnrffunction;

prefix extnrf3gpp;

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

description "This IOC represents external NRF function controlled by another management domain.";

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-11 {

description "Ericsson refactoring.";

}

grouping ExternalNRFFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

}

augment "/subnet3gpp:SubNetwork" {

list ExternalNRFFunction {

description "5G Core External NRF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalNRFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;    }

}

}

### H.5.9	module _3gpp-5gc-nrm-externalnssffunction@2019-10-28.yang

module _3gpp-5gc-nrm-externalnssffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-externalnssffunction;

prefix extnssf3gpp;

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

description "This IOC represents external NSSF function controlled by another management domain.";

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-11 {

description "Ericsson refactoring.";

}

grouping ExternalNSSFFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

}

augment "/subnet3gpp:SubNetwork" {

list ExternalNSSFFunction {

description "5G Core External NSSF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalNSSFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## H.5.10	module _3gpp-5gc-nrm-lmffunction@2019-10-25.yang

module _3gpp-5gc-nrm-lmffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-lmffunction;

prefix lmf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

description "This IOC represents the LMF function defined in 3GPP TS 23.501.";

reference "3GPP TS 28.541";

revision 2019-10-25 { reference "S5-194457 S5193518"; }

revision 2019-05-15 {

description "initial revision";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

grouping LMFFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

list managedNFProfile {

key idx;

min-elements 1;

uses types3gpp:ManagedNFProfile;

}

list commModelList {

min-elements 1;

key "groupId";

uses types5g3gpp:CommModel;

}

}

augment "/me3gpp:ManagedElement" {

list LMFFunction {

description "5G Core LMF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses LMFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## H.5.11	module _3gpp-5gc-nrm-n3iwffunction@2019-10-28.yang

module _3gpp-5gc-nrm-n3iwffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-n3iwffunction;

prefix n3iwf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

description "This IOC represents the N3IWF function which is used to enable non-3GPP

access networks connected to the 5GC. For more information about the N3IWF, see 3GPP TS 23.501.";

reference "3GPP TS 28.541";

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-05-22 {

description "initial revision";

}

grouping N3IWFFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

list commModelList {

min-elements 1;

key "groupId";

uses types5g3gpp:CommModel;

}

}

augment "/me3gpp:ManagedElement" {

list N3IWFFunction {

description "5G Core N3IWF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses N3IWFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## H.5.12	module _3gpp-5gc-nrm-nfprofile@2019-06-17.yang

module _3gpp-5gc-nrm-nfprofile {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-nfprofile;

prefix nfp3gpp;

import _3gpp-common-yang-types { prefix types3gpp; }

import ietf-inet-types { prefix inet; }

import ietf-yang-types { prefix yang; }

import _3gpp-5gc-nrm-nfservice { prefix nfs3gpp; }

organization "3gpp SA5";

description "NF profile class.";

reference "3GPP TS 29.510";

revision 2019-06-17 {

description "initial revision";

}

grouping NFProfileGrp {

leaf nfInstanceID {

description "String uniquely identifying a NF instance.";

mandatory true;

type string;

}

leaf nfType {

description "Type of Network Function.";

mandatory true;

type types3gpp:NfType;

}

leaf nfStatus {

description "Status of the NF Instance.";

mandatory true;

type NFStatus;

}

leaf heartBeatTimer {

description "Time in seconds expected between 2 consecutive heart-beat messages from

an NF Instance to the NRF. It may be included in the registration request.

When present in the request it shall contain the heartbeat time proposed by the NF service consumer.";

//conditional support

type uint16;

}

list plmnList {

description "PLMN(s) of the Network Function.

This IE shall be present if this information is available for the NF.

If not provided, PLMN ID(s) of the PLMN of the NRF are assumed for the NF.";

//conditional support

min-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

list sNssais { //is the key unique

description "S-NSSAIs of the Network Function. If not provided, the NF can serve any S-NSSAI.

When present this IE represents the list of S-NSSAIs supported in all the PLMNs listed in the plmnList IE.";

min-elements 1;

//optional support

key "sst sd";

uses Snssai;

}

list perPlmnSnssaiList {

description "This IE may be included when the list of S-NSSAIs supported by the NF for each PLMN it is supporting is different.

When present, this IE shall include the S-NSSAIs supported by the Network Function

for each PLMN supported by the Network Function. When present, this IE shall override sNssais IE.";

min-elements 1;

//optional support

key idx; //no obvious leaf to use as a key

leaf idx { type uint32; }

uses PlmnSnssai;

}

leaf-list nsiList {

description "NSI identities of the Network Function.

If not provided, the NF can serve any NSI.";

//optional support

min-elements 1;

type string;

}

leaf fqdn {

description "FQDN of the Network Function. For AMF, the FQDN registered with the NRF

shall be that of the AMF Name.";

//conditional support

type inet:domain-name;

}

leaf interPlmnFqdn {

description "If the NF needs to be discoverable by other NFs in a different PLMN,

then an FQDN that is used for inter-PLMN routing is specified.";

//conditional support

type inet:domain-name;

}

leaf-list ipv4Addresses {

description "IPv4 address(es) of the Network Function.";

min-elements 1;

//conditional support

type inet:ipv4-address;

}

leaf-list ipv6Addresses {

description "IPv6 address(es) of the Network Function.";

min-elements 1;

//conditional support

type inet:ipv6-address;

}

list allowedPlmns {

description "PLMNs allowed to access the NF instance.

If not provided, any PLMN is allowed to access the NF.";

min-elements 1;

//optional support

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf-list allowedNfTypes {

description "Type of the NFs allowed to access the NF instance.

If not provided, any NF type is allowed to access the NF.";

min-elements 1;

//optional support

type types3gpp:NfType;

}

leaf-list allowedNfDomains {

description "Pattern representing the NF domain names allowed to access the NF instance.

If not provided, any NF domain is allowed to access the NF.";

min-elements 1;

//optional support

type string;

}

list allowedNssais { //is the key unique

description "S-NSSAI of the allowed slices to access the NF instance.

If not provided, any slice is allowed to access the NF.";

min-elements 1;

//optional support

key "sst sd";

uses Snssai;

}

leaf priority {

description "Priority (relative to other NFs of the same type) in the range of 0-65535, to be used for NF selection;

lower values indicate a higher priority. If priority is also present in the nfServiceList parameters,

those will have precedence over this value. The NRF may overwrite the received priority value when exposing

an NFProfile with the Nnrf_NFDiscovery service.";

//optional support

type uint16;

}

leaf capacity {

description "Static capacity information in the range of 0-65535, expressed as a weight

relative to other NF instances of the same type; if capacity is also present

in the nfServiceList parameters, those will have precedence over this value.";

//optional support

type uint16;

}

leaf load {

description "Dynamic load information, ranged from 0 to 100, indicates the current load percentage of the NF.";

//optional support

type types3gpp:Load;

}

leaf locality {

description "Operator defined information about the location of the NF instance (e.g. geographic location, data center).";

//optional support

type string;

}

grouping udrInfo {

//optional support

leaf groupId {

description "Identity of the UDR group that is served by the UDR instance.

If not provided, the UDR instance does not pertain to any UDR group.";

//optional support

type string;

}

list supiRanges {

description "List of ranges of SUPI's whose profile data is available in the UDR instance.";

key "start end pattern";

min-elements 1;

//optional support

uses SupiRange;

}

list gpsiRanges {

description "List of ranges of GPSIs whose profile data is available in the UDR instance.";

key "start end pattern";

min-elements 1;

//optional support

uses IdentityRange;

}

list externalGroupIdentifiersRanges {

description "List of ranges of external groups whose profile data is available in the UDR instance.";

key "start end pattern";

min-elements 1;

//optional support

uses IdentityRange;

}

leaf-list supportedDataSets {

description "List of supported data sets in the UDR instance.

If not provided, the UDR supports all data sets.";

min-elements 1;

//optional support

type DataSetId;

}

}

grouping udmInfo {

//optional support

leaf groupId {

description "Identity of the UDM group that is served by the UDM instance.

If not provided, the UDM instance does not pertain to any UDM group.";

//optional support

type string;

}

list supiRanges {

description "List of ranges of SUPI's whose profile data is available in the UDM instance.";

key "start end pattern";

min-elements 1;

//optional support

uses SupiRange;

}

list gpsiRanges {

description "List of ranges of GPSIs whose profile data is available in the UDM instance.";

key "start end pattern";

min-elements 1;

//optional support

uses IdentityRange;

}

list externalGroupIdentifiersRanges {

description "List of ranges of external groups whose profile data is available in the UDM instance.";

key "start end pattern";

min-elements 1;

//optional support

uses IdentityRange;

}

leaf-list routingIndicators {

description "List of Routing Indicator information that allows to route network signalling with SUCI

to the UDM instance. If not provided, the UDM can serve any Routing Indicator.

Pattern: '^[0-9]{1,4}$'.";

//optional support

min-elements 1;

type string;

}

}

grouping ausfInfo {

//optional support

leaf groupId {

description "Identity of the AUSF group. If not provided, the AUSF instance does not pertain to any AUSF group.";

//optional support

type string;

}

list supiRanges {

description "List of ranges of SUPIs that can be served by the AUSF instance. If not provided, the AUSF can serve any SUPI.";

key "start end pattern";

min-elements 1;

//optional support

uses SupiRange;

}

leaf-list routingIndicators {

description "List of Routing Indicator information that allows to route network signalling with SUCI

to the AUSF instance. If not provided, the AUSF can serve any Routing Indicator.

Pattern: '^[0-9]{1,4}$'.";

//optional support

min-elements 1;

type string;

}

}

grouping amfInfo {

//optional support

leaf amfRegionId {

description "AMF region identifier";

type string;

}

leaf amfSetId {

description "AMF set identifier";

type string;

}

list guamiList {

description "List of supported GUAMIs.";

key idx; //no obvious leaf to use as a key

leaf idx { type uint32; }

min-elements 1;

uses Guami;

}

list taiList {

description "The list of TAIs the AMF can serve. It may contain the non-3GPP access TAI.

The absence of this attribute and the taiRangeList attribute indicate that

the AMF can be selected for any TAI in the serving network.";

key idx; //no obvious leaf to use as a key

leaf idx { type uint32; }

//optional support

min-elements 1;

uses Tai;

}

list taiRangeList {

description "The range of TAIs the AMF can serve. The absence of this attribute and the taiList

attribute indicate that the AMF can be selected for any TAI in the serving network.";

//optional support

min-elements 1;

key idx; //no obvious leaf to use as a key

leaf idx { type uint32; }

uses TaiRange;

}

list backupInfoAmfFailure {

description "List of GUAMIs for which the AMF acts as a backup for AMF failure.";

key idx; //no obvious leaf to use as a key

leaf idx { type uint32; }

//optional support

min-elements 1;

uses Guami;

}

list backupInfoAmfRemoval {

description "List of GUAMIs for which the AMF acts as a backup for planned AMF removal.";

key idx; //no obvious leaf to use as a key

leaf idx { type uint32; }

//optional support

min-elements 1;

uses Guami;

}

list n2InterfaceAmfInfo {

description "N2 interface information of the AMF. This information needs not be sent in NF Discovery responses.

It may be used by the NRF to update the DNS for AMF discovery by the 5G Access Network.";

//optional support

max-elements 1;

key idx; //no obvious leaf to use as a key

leaf idx { type uint32; }

uses N2InterfaceAmfInfo;

}

}

grouping smfInfo {

//optional support

list sNssaiSmfInfoList {

description "List of parameters supported by the SMF per S-NSSAI.";

min-elements 1;

key idx; //no obvious leaf to use as a key

leaf idx { type uint32; }

uses sNssaiSmfInfoItem;

}

list taiList {

description "The list of TAIs the SMF can serve. It may contain the non-3GPP access TAI.

The absence of this attribute and the taiRangeList attribute indicate that

the SMF can be selected for any TAI in the serving network.";

key idx; //no obvious leaf to use as a key

leaf idx { type uint32; }

//optional support

min-elements 1;

uses Tai;

}

list taiRangeList {

description "The range of TAIs the SMF can serve. The absence of this attribute and the taiList

attribute indicate that the SMF can be selected for any TAI in the serving network.";

//optional support

min-elements 1;

key idx; //no obvious leaf to use as a key

leaf idx { type uint32; }

uses TaiRange;

}

leaf pgwFqdn {

description "The FQDN of the PGW if the SMF is a combined SMF/PGW-C.";

//optional support

type inet:domain-name;

}

leaf-list accessType {

description "If included, this IE shall contain the access type (3GPP_ACCESS and/or NON_3GPP_ACCESS) supported by the SMF.

If not included, it shall be assumed the both access types are supported.";

//conditional support

min-elements 1;

max-elements 2;

type AccessType;

}

}

grouping upfInfo {

//optional support

list sNssaiUpfInfoList {

description "List of parameters supported by the UPF per S-NSSAI.";

min-elements 1;

key idx; //no obvious leaf to use as a key

leaf idx { type uint32; }

uses SnssaiUpfInfoItem;

}

leaf-list smfServingArea {

description "The SMF service area(s) the UPF can serve.

If not provided, the UPF can serve any SMF service area.";

//optional support

min-elements 1;

type string;

}

list interfaceUpfInfo {

description "List of User Plane interfaces configured on the UPF. When this IE is provided in the NF Discovery response,

the NF Service Consumer (e.g. SMF) may use this information for UPF selection.";

key idx; //no obvious leaf to use as a key

leaf idx { type uint32; }

//optional support

min-elements 1;

uses InterfaceUpfInfoItem;

}

leaf iwkEpsInd {

description "Indicates whether interworking with EPS is supported by the UPF.

true: Supported

false (default): Not Supported";

//optional support

type boolean;

}

leaf-list pduSessionTypes {

description "List of PDU session type(s) supported by the UPF. The absence of this attribute indicates that the UPF can be selected

for any PDU session type.";

//optional support

min-elements 1;

type PduSessionType;

}

}

grouping pcfInfo {

//optional support

leaf-list dnnList {

description "DNNs supported by the PCF.

If not provided, the PCF can serve any DNN.";

//optional support

min-elements 1;

type string;

}

list supiRanges {

description "List of ranges of SUPIs that can be served by the PCF instance. If not provided, the PCF can serve any SUPI.";

key "start end pattern";

min-elements 1;

//optional support

uses SupiRange;

}

leaf rxDiamHost {

description "This IE shall be present if the PCF supports Rx interface.

When present, this IE shall indicate the Diameter host of the Rx interface for the PCF.

Pattern: '^([A-Za-z0-9]+(-[A-Za-z0-9]+).)+[a-z]{2,}$'.";

//conditional support

type string;

}

leaf rxDiamRealm {

description "This IE shall be present if the PCF supports Rx interface.

When present, this IE shall indicate the Diameter realm of the Rx interface for the PCF.

Pattern: '^([A-Za-z0-9]+(-[A-Za-z0-9]+).)+[a-z]{2,}$'.";

//conditional support

type string;

}

}

grouping bsfInfo {

//optional support

list ipv4AddressRanges {

description "List of ranges of IPv4 addresses handled by BSF.

If not provided, the BSF can serve any IPv4 address.";

//optional support

key "start end";

uses types3gpp:Ipv4AddressRange;

}

leaf-list dnnList {

description "List of DNNs handled by the BSF

If not provided, the BSF can serve any DNN.";

//optional support

min-elements 1;

type string;

}

leaf-list ipDomainList {

description "List of IPv4 address domains, as described in subclause 6.2 of 3GPP TS 29.513, handled by the BSF.

If not provided, the BSF can serve any IP domain.";

//optional support

min-elements 1;

type string;

}

list ipv6PrefixRanges {

description "List of ranges of IPv6 prefixes handled by the BSF.

If not provided, the BSF can serve any IPv6 prefix.";

//optional support

key "start end";

uses types3gpp:Ipv6PrefixRange;

}

}

grouping chfInfo {

//optional support

list supiRangeList {

description "List of ranges of SUPIs that can be served by the CHF instance. If not provided, the CHF can serve any SUPI.";

key "start end pattern";

min-elements 1;

//optional support

uses SupiRange;

}

list gpsiRangeList {

description "List of ranges of GPSI that can be served by the CHF instance. If not provided, the CHF can serve any GPSI.";

key "start end pattern";

min-elements 1;

//optional support

uses IdentityRange;

}

list plmnRangeList {

description "List of ranges of PLMNs (including the PLMN IDs of the CHF instance) that can be served by the CHF instance.

If not provided, the CHF can serve any PLMN.";

min-elements 1;

//optional support

key "mcc mnc";

uses types3gpp:PLMNId;

}

}

grouping nrfInfoGrp {

//optional support

list servedUdrInfo {

description "This attribute contains all the udrInfo attributes locally configured in the NRF or the NRF received during NF registration.";

//optional support

key nfInstanceID;

leaf nfInstanceID {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses udrInfo;

}

list servedUdmInfo {

description "This attribute contains all the udmInfo attributes locally configured in the NRF or the NRF received during NF registration.";

//optional support

key nfInstanceID;

leaf nfInstanceID {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses udmInfo;

}

list servedAusfInfo {

description "This attribute contains all the ausfInfo attributes locally configured in the NRF or the NRF received during NF registration.";

//optional support

key nfInstanceID;

leaf nfInstanceID {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses ausfInfo;

}

list servedAmfInfo {

description "This attribute contains all the amfInfo attributes locally configured in the NRF or the NRF received during NF registration.";

//optional support

key nfInstanceID;

leaf nfInstanceID {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses amfInfo;

}

list servedSmfInfo {

description "This attribute contains all the smfInfo attributes locally configured in the NRF or the NRF received during NF registration.";

//optional support

key nfInstanceID;

leaf nfInstanceID {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses smfInfo;

}

list servedUpfInfo {

description "This attribute contains all the upfInfo attributes locally configured in the NRF or the NRF received during NF registration.";

//optional support

key nfInstanceID;

leaf nfInstanceID {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses upfInfo;

}

list servedPcfInfo {

description "This attribute contains all the pcfInfo attributes locally configured in the NRF or the NRF received during NF registration.";

//optional support

key nfInstanceID;

leaf nfInstanceID {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses pcfInfo;

}

list servedBsfInfo {

description "This attribute contains all the bsfInfo attributes locally configured in the NRF or the NRF received during NF registration.";

//optional support

key nfInstanceID;

leaf nfInstanceID {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses bsfInfo;

}

list servedChfInfo {

description "This attribute contains all the bsfInfo attributes locally configured in the NRF or the NRF received during NF registration.";

//optional support

key nfInstanceID;

leaf nfInstanceID {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses chfInfo;

}

}

list nrfInfo {

key idx; //no obvious leaf to use as a key

leaf idx { type uint32; }

max-elements 1;

uses nrfInfoGrp;

}

leaf customInfo {

description "Specific data for custom Network Functions.";

type string;

}

leaf recoveryTime {

description "Timestamp when the NF was (re)started.";

//optional support

type yang:date-and-time;

}

leaf nfServicePersistence {

description "If present, and set to true, it indicates that the different service instances of a same NF Service in this NF instance,

supporting a same API version, are capable to persist their resource state in shared storage and therefore these resources

are available after a new NF service instance supporting the same API version is selected by a NF Service Consumer (see 3GPP TS 23.527).

Otherwise, it indicates that the NF Service Instances of a same NF Service are not capable to share resource state inside the NF Instance.";

//optional support

type boolean;

}

list nfServices {

description "List of NF Service Instances. It shall include the services produced by the NF that can be discovered by other NFs.";

key serviceInstanceID;

//optional support

min-elements 1;

uses nfs3gpp:NFServiceGrp;

}

leaf nfProfileChangesSupportInd {

description "NF Profile Changes Support Indicator. This IE may be present in the NFRegister or NFUpdate (NF Profile Complete Replacement) request and shall be absent in the response.

true: the NF Service Consumer supports receiving NF Profile Changes in the response.

false (default): the NF Service Consumer does not support receiving NF Profile Changes in the response.";

//optional support

type boolean;

}

leaf nfProfileChangesInd {

description "NF Profile Changes Indicator. This IE shall be absent in the request to the NRF and may be included by the NRF in NFRegister or NFUpdate (NF Profile Complete Replacement) response.

true: the NF Profile contains NF Profile changes.

false (default): complete NF Profile.";

//optional support

type boolean;

}

list defaultNotificationSubscriptions {

description "Notification endpoints for different notification types.";

key notificationType;

//optional support

min-elements 1;

uses types3gpp:DefaultNotificationSubscription;

}

}

typedef NFStatus {

type enumeration {

enum REGISTERED;

enum SUSPENDED;

}

}

typedef DataSetId {

type enumeration {

enum SUBSCRIPTION;

enum POLICY;

enum EXPOSURE;

enum APPLICATION;

}

}

grouping SupiRange {

leaf start {

description "First value identifying the start of a SUPI range. To be used when the range of SUPI's can be represented as a numeric range (e.g., IMSI ranges).";

type string {

pattern '^[0-9]+$';

}

}

leaf end {

description "Last value identifying the end of a SUPI range. To be used when the range of SUPI's can be represented as a numeric range (e.g. IMSI ranges).";

type string {

pattern '^[0-9]+$';

}

}

leaf pattern {

description "Pattern representing the set of SUPI's belonging to this range.

A SUPI value is considered part of the range if and only if the SUPI string fully matches the regular expression.";

type string;

}

}

grouping IdentityRange {

leaf start {

description "First value identifying the start of an identity range. To be used when the range of identities can be represented as a numeric range (e.g., MSISDN ranges).";

type string {

pattern '^[0-9]+$';

}

}

leaf end {

description "Last value identifying the end of an identity range. To be used when the range of identities can be represented as a numeric range (e.g. MSISDN ranges).";

type string {

pattern '^[0-9]+$';

}

}

leaf pattern {

description "Pattern representing the set of identities belonging to this range.

An identity value is considered part of the range if and only if the identity string fully matches the regular expression.";

type string;

}

}

grouping TacRange {

leaf start {

description "First value identifying the start of a TAC range, to be used when the range of TAC's can be represented as a hexadecimal range (e.g., TAC ranges).";

type string {

pattern '^([A-Fa-f0-9]{4}|[A-Fa-f0-9]{6}$)';

}

}

leaf end {

description "Last value identifying the end of a TAC range, to be used when the range of TAC's can be represented as a hexadecimal range (e.g. TAC ranges).";

type string {

pattern '^([A-Fa-f0-9]{4}|[A-Fa-f0-9]{6})$';

}

}

leaf pattern {

description "Pattern representing the set of TAC's belonging to this range.";

type string;

}

}

grouping SnssaiUpfInfoItem {

list sNssai { //is the key unique

description "Supported S-NSSAI.";

min-elements 1;

max-elements 1;

key "sst sd";

uses Snssai;

}

list dnnUpfInfoList {

description "List of parameters supported by the UPF per DNN.";

min-elements 1;

key dnn;

uses DnnUpfInfoItem;

}

}

grouping DnnUpfInfoItem {

leaf dnn {

description "String representing a Data Network.";

mandatory true;

type string;

}

leaf-list dnaiList {

description "List of Data network access identifiers supported by the UPF for this DNN.

The absence of this attribute indicates that the UPF can be selected for this DNN for any DNAI.";

min-elements 1;

type string; //dnai is the type but its only a string with desc: DNAI (Data network access identifier), is this needed as its own typedef or string is ok

}

leaf-list pduSessionTypes {

description "List of PDU session type(s) supported by the UPF for a specific DNN.";

min-elements 1;

type PduSessionType;

}

}

grouping Snssai {

leaf sst {

description "Unsigned integer, within the range 0 to 255, representing the Slice/Service Type.

It indicates the expected Network Slice behaviour in terms of features and services.";

mandatory true;

type uint32;

}

leaf sd {

description "3-octet string, representing the Slice Differentiator, in hexadecimal representation.";

//optional

type string {

pattern '^[A-Fa-f0-9]{6}$';

}

}

reference "3GPP TS 29.571";

}

typedef PduSessionType {

type enumeration {

enum IPV4;

enum IPV6;

enum IPV4V6;

enum UNSTRUCTURED;

enum ETHERNET;

}

}

grouping Guami {

list plmnId {

description "PLMN Identity.";

min-elements 1;

max-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

list amfId {

description "AMF Identity.";

min-elements 1;

max-elements 1;

key "amfRegionId amfSetId amfPointer";

uses types3gpp:AmfIdentifier;

}

}

grouping Tai {

list plmnId {

description "PLMN Identity.";

min-elements 1;

max-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf tac { type types3gpp:Tac; }

}

grouping InterfaceUpfInfoItem {

leaf interfaceType {

description "User Plane interface type.";

mandatory true;

type UPInterfaceType;

}

////At least one of the addressing parameters (ipv4address, ipv6adress or endpointFqdn) shall be included in the InterfaceUpfInfoItem.

choice address {

case ipv4EndpointAddresses {

leaf-list ipv4EndpointAddresses {

description "Available endpoint IPv4 address(es) of the User Plane interface.";

//conditional support

min-elements 1;

type inet:ipv4-address;

}

}

case ipv6EndpointAddresses {

leaf-list ipv6EndpointAddresses {

description "Available endpoint IPv6 address(es) of the User Plane interface.";

//conditional support

min-elements 1;

type inet:ipv6-address;

}

}

case endpointFqdn {

leaf endpointFqdn {

description "FQDN of available endpoint of the User Plane interface.";

//conditional support

type inet:domain-name;

}

}

}

leaf networkInstance {

description "Network Instance associated to the User Plane interface.";

//optional support

type string;

}

}

typedef UPInterfaceType {

type enumeration {

enum N3;

enum N6;

enum N9;

}

}

grouping TaiRange {

list plmnId {

description "PLMN ID related to the TacRange.";

min-elements 1;

max-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

list tacRangeList { //is this key unique

description "The range of the TACs.";

min-elements 1;

key "start end";

uses TacRange;

}

}

typedef AccessType {

type enumeration {

enum 3GPP_ACCESS;

enum NON_3GPP_ACCESS;

}

}

grouping N2InterfaceAmfInfo {

//At least one of the addressing parameters (ipv4address or ipv6adress) shall be included.

choice address {

case ipv4EndpointAddress {

leaf-list ipv4EndpointAddress {

description "Available AMF endpoint IPv4 address(es) for N2.";

//conditional support

min-elements 1;

type inet:ipv4-address;

}

}

case ipv6EndpointAddress {

leaf-list ipv6EndpointAddress {

description "Available AMF endpoint IPv6 address(es) for N2.";

//conditional support

min-elements 1;

type inet:ipv6-address;

}

}

}

leaf amfName {

description "AMF name.";

type string;

}

}

grouping sNssaiSmfInfoItem {

list sNssai { //is the key unique

description "Supported S-NSSAI.";

min-elements 1;

max-elements 1;

key "sst sd";

uses Snssai;

}

list dnnSmfInfoList { //is the key unique

description "List of parameters supported by the SMF per DNN.";

min-elements 1;

key dnn;

uses DnnSmfInfoItem;

}

}

grouping DnnSmfInfoItem {

leaf dnn {

description "Supported DNN.";

mandatory true;

type string;

}

}

grouping PlmnSnssai {

list plmnId {

description "PLMN ID for which list of supported S-NSSAI(s) is provided.";

min-elements 1;

max-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

list sNssaiList { //is the key unique

description "The specific list of S-NSSAIs supported by the given PLMN.";

min-elements 1;

key "sst sd";

uses Snssai;

}

}

}

## H.5.13	module _3gpp-5gc-nrm-nfservice.yang

<CODE BEGINS>

module _3gpp-5gc-nrm-nfservice {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-nfservice;

prefix nfs3gpp;

import _3gpp-common-yang-types { prefix types3gpp; }

import ietf-yang-types { prefix yang; }

import ietf-inet-types { prefix inet; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "NF service class.";

reference "3GPP TS 29.510";

revision 2021-01-25 { reference CR-0454 ; }

revision 2020-11-05 { reference CR-0412 ; }

revision 2019-06-17 { reference "initial revision"; }

grouping NFServiceGrp {

description "Represents the NFService IOC";

leaf serviceInstanceID {

description

"Unique ID of the service instance within a given NF Instance.";

mandatory true;

type string;

}

leaf serviceName {

description "Name of the service instance (e.g. 'nudm-sdm').";

mandatory true;

type ServiceName;

}

list versions { //check in review if key is ok (unique)

description "API versions supported by the NF Service and if available,

the corresponding retirement date of the NF Service.";

min-elements 1;

key "apiVersionInUri apiFullVersion";

uses NFServiceVersion;

}

leaf scheme {

description "URI scheme (e.g. 'http', 'https').";

mandatory true;

type UriScheme;

}

leaf nfServiceStatus {

description "Status of the NF Service Instance.";

mandatory true;

type NFServiceStatus;

}

leaf fqdn {

description "FQDN of the NF Service Instance.";

//optional support

type inet:domain-name;

}

leaf interPlmnFqdn {

description "If the NF service needs to be discoverable by other NFs in a

different PLMN, then an FQDN that is used for inter PLMN routing.";

//optional support

type inet:domain-name;

}

list ipEndPoints {

description "IP address(es) and port information of the Network Function

(including IPv4 and/or IPv6 address)where the service is listening

for incoming service requests.";

//optional support

key idx;

leaf idx {

type string;

}

min-elements 1;

uses ipEndPoint;

}

leaf apiPrefix {

description "Optional path segment(s) used to construct the {apiRoot}

variable of the different API URIs.";

//optional support

type string;

}

list defaultNotificationSubscriptions {

description "Notification endpoints for different notification types.";

key notificationType;

//optional support

min-elements 1;

uses types3gpp:DefaultNotificationSubscription;

}

list allowedPlmns {

description "PLMNs allowed to access the service instance.

The absence of this attribute indicates that any PLMN is allowed to

access the service instance.";

min-elements 1;

//optional support

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf-list allowedNfTypes {

description "Type of the NFs allowed to access the service instance.

The absence of this attribute indicates that any NF type is allowed

to access the service instance.";

min-elements 1;

//optional support

type types3gpp:NfType;

}

leaf-list allowedNfDomains {

description "Pattern representing the NF domain names allowed to access the service instance.";

//optional support

min-elements 1;

type string;

}

list allowedNssais {

description "S-NSSAI of the allowed slices to access the service instance.

The absence of this attribute indicates that any slice is allowed to

access the service instance.";

min-elements 1;

//optional support

key "sd sst";

uses types5g3gpp:SNssai;

}

leaf priority {

description "Priority (relative to other services of the same type)

in the range of 0-65535, to be used for NF Service selection; lower

values indicate a higher priority.";

//optional support

type uint16;

}

leaf capacity {

description "Static capacity information in the range of 0-65535,

expressed as a weight relative to other services of the same type.";

//optional support

type uint16;

}

leaf load {

description "Dynamic load information, ranged from 0 to 100, indicates

the current load percentage of the NF Service.";

//optional support

type types3gpp:Load;

}

leaf recoveryTime {

description "Timestamp when the NF was (re)started.";

//optional support

type yang:date-and-time;

}

list chfServiceInfo { //is the key unique

description "Specific data for a CHF service instance.";

//optional support

max-elements 1;

key "primaryChfServiceInstance secondaryChfServiceInstance";

uses ChfServiceInfo;

}

leaf supportedFeatures {

description "Supported Features of the NF Service instance.";

//optional support

type SupportedFeatures;

}

}

typedef SupportedFeatures {

type string {

pattern '[A-Fa-f0-9]*';

}

}

grouping ipEndPoint {

choice address {

leaf ipv4Address {

type inet:ipv4-address;

}

leaf ipv6Address {

type inet:ipv6-address;

}

leaf ipv6Prefix {

type inet:ipv6-prefix;

}

}

leaf transport {

type TransportProtocol;

}

leaf port {

type uint16;

}

}

typedef TransportProtocol {

type enumeration {

enum TCP;

enum STCP;

enum UDP;

}

}

grouping NFServiceVersion {

leaf apiVersionInUri {

mandatory true;

type string;

}

leaf apiFullVersion {

mandatory true;

type string;

}

leaf expiry {

//optional to support

type yang:date-and-time;

}

}

typedef ServiceName {

type enumeration {

enum NNRF_NFM;

enum NNRF_DISC;

enum NUDM_SDM;

enum NUDM_UECM;

enum NUDM_UEAU;

enum NUDM_EE;

enum NUDM_PP;

enum NAMF_COMM;

enum NAMF_EVTS;

enum NAMF_MT;

enum NAMF_LOC;

enum NSMF_PDUSESSION;

enum NSMF_EVENT-EXPOSURE;

enum NAUSF_AUTH;

enum NAUSF_SORPROTECTION;

enum NNEF_PFDMANAGEMENT;

enum NPCF_AM-POLICY-CONTROL;

enum NPCF_SMPOLICYCONTROL;

enum NPCF_POLICYAUTHORIZATION;

enum NPCF_BDTPOLICYCONTROL;

enum NPCF_EVENTEXPOSURE;

enum NPCF_UE_POLICY_CONTROL;

enum NSMSF_SMS;

enum NNSSF_NSSELECTION;

enum NNSSF_NSSAIAVAILABILITY;

enum NUDR_DR;

enum NLMF_LOC;

enum N5G_EIR_EIC;

enum NBSF_MANAGEMENT;

enum NCHF_SPENDINGLIMITCONTROL;

enum NCHF_CONVERGEDCHARGING;

enum NNWDAF_EVENTSSUBSCRIPTION;

enum NNWDAF_ANALYTICSINFO;

}

}

typedef UriScheme {

type enumeration {

enum HTTP;

enum HTTPS;

}

}

typedef NFServiceStatus {

type enumeration {

enum REGISTERED;

enum SUSPENDED;

enum UNDISCOVERABLE;

}

}

grouping ChfServiceInfo {

leaf primaryChfServiceInstance {

description "Shall be present if the CHF service instance serves as a

secondary CHF instance of another primary CHF service instance.";

//conditional to support

type string;

}

leaf secondaryChfServiceInstance {

description "Shall be present if the CHF service instance serves as a

primary CHF instance of another secondary CHF service instance.";

//conditional to support

type string;

}

}

}

<CODE ENDS>

## H.5.14	module _3gpp-5gc-nrm-ngeirfunction.yang

module _3gpp-5gc-nrm-ngeirfunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-ngeirfunction;

prefix ngeir3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the 5G-EIR function in 5GC. For more

information about the 5G-EIR, see 3GPP TS 23.501.";

reference "3GPP TS 28.541";

revision 2020-11-05 { reference CR-0412 ; }

revision 2019-10-25 { reference "S5-194457 S5-195427 S5-193518"; }

revision 2019-05-15 {reference "initial revision"; }

grouping NGEIRFunctionGrp {

description "Represents the NGEIRFunction IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at

least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and

a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//optional support

reference "3GPP TS 23.003";

key "sd sst";

uses types5g3gpp:SNssai;

}

list managedNFProfile {

key idx;

min-elements 1;

max-elements 1;

uses types3gpp:ManagedNFProfile;

}

list commModelList {

min-elements 1;

key "groupId";

description "Specifies a list of commModel. It can be used by NF and

NF services to interact with each other in 5G Core network ";

reference "3GPP TS 23.501";

uses types5g3gpp:CommModel;

}

}

augment "/me3gpp:ManagedElement" {

list NGEIRFunction {

description "5G Core NGEIR Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NGEIRFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## H.5.15	module _3gpp-5gc-nrm-nrffunction.yang

module _3gpp-5gc-nrm-nrffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-nrffunction;

prefix nrf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5gc-nrm-nfprofile { prefix nfp3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the NRF function in 5GC.

For more information about the NRF, see 3GPP TS 23.501 [2].";

reference "3GPP TS 28.541";

revision 2020-11-05 { reference CR-0412 ; }

revision 2020-08-03 { reference "CR-0321"; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-05-15 { reference "initial revision"; }

grouping NRFFunctionGrp {

description "Represents the NRFFunction IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at

least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a

Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based

interface.";

type inet:domain-name;

}

leaf-list cNSIIdList {

description "NSI ID. NSI ID is an identifier for identifying the Core

Network part of a Network Slice instance when multiple Network Slice

instances of the same Network Slice are deployed, and there is a need

to differentiate between them in the 5GC, see clause 3.1 of TS 23.501

and subclause 6.1.6.2.7 of 3GPP TS 29.531";

type string;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//optional support

reference "3GPP TS 23.003";

key "sd sst";

uses types5g3gpp:SNssai;

}

list nFProfileList {

description "Set of NFProfile(s) to be registered in the NRF instance.";

//optional support

key nfInstanceID;

uses nfp3gpp:NFProfileGrp;

}

}

augment "/me3gpp:ManagedElement" {

list NRFFunction {

description "5G Core NRF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NRFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## H.5.16	module _3gpp-5gc-nrm-nssffunction.yang

module _3gpp-5gc-nrm-nssffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-nssffunction;

prefix nssf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the NSSF function in 5GC. For more

information about the NSSF, see 3GPP TS 23.501.";

reference "3GPP TS 28.541";

revision 2020-11-05 { reference CR-0412 ; }

revision 2020-08-03 { reference "CR-0321"; }

revision 2019-10-25 { reference "S5-194457 S5-195427 S5-193518"; }

revision 2019-05-15 { reference "initial revision"; }

grouping NSSFFunctionGrp {

description "Represents the NSSFFunction IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least

one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a

Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based

interface.";

type inet:domain-name;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

reference "3GPP TS 23.003";

key "sd sst";

uses types5g3gpp:SNssai;

}

leaf-list cNSIIdList {

description "NSI ID. NSI ID is an identifier for identifying the Core

Network part of a Network Slice instance when multiple Network Slice

instances of the same Network Slice are deployed, and there is a need

to differentiate between them in the 5GC, see clause 3.1 of TS 23.501

and subclause 6.1.6.2.7 of 3GPP TS 29.531";

type string;

}

list managedNFProfile {

key idx;

min-elements 1;

max-elements 1;

uses types3gpp:ManagedNFProfile;

}

}

augment "/me3gpp:ManagedElement" {

list NSSFFunction {

description "5G Core NSSF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NSSFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## H.5.17	module _3gpp-5gc-nrm-nwdaffunction.yang

module _3gpp-5gc-nrm-nwdaffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-nwdaffunction;

prefix nwdaf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the NWDAF function in 5GC. For more information about the NWDAF, see 3GPP TS 23.501.";

reference "3GPP TS 28.541";

revision 2020-11-05 { reference CR-0412 ; }

revision 2019-10-25 { reference "S5-194457 S5-195427 S5-193518"; }

revision 2019-05-15 {reference "initial revision"; }

grouping NWDAFFunctionGrp {

description "Represents the NWDAFFunction IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at

least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a

Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based

interface.";

type inet:domain-name;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//optional support

reference "3GPP TS 23.003";

key "sd sst";

uses types5g3gpp:SNssai;

}

list managedNFProfile {

key idx;

min-elements 1;

max-elements 1;

uses types3gpp:ManagedNFProfile;

}

list commModelList {

min-elements 1;

key "groupId";

description "Specifies a list of commModel. It can be used by NF and

NF services to interact with each other in 5G Core network ";

reference "3GPP TS 23.501";

uses types5g3gpp:CommModel;

}

}

augment "/me3gpp:ManagedElement" {

list NWDAFFunction {

description "5G Core NWDAF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NWDAFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## H.5.18	module _3gpp-5gc-nrm-pcffunction.yang

module _3gpp-5gc-nrm-pcffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-pcffunction;

prefix pcf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the PCF function in 5GC. For more

information about the PCF, see 3GPP TS 23.501.";

reference "3GPP TS 28.541";

revision 2020-11-05 { reference CR-0412 ; }

revision 2020-08-06 { reference "CR-0333"; }

revision 2020-08-06 { reference "CR-0331"; }

revision 2019-10-25 { reference "S5-194457 S5-193518"; }

revision 2019-05-22 { reference "initial revision"; }

grouping PCFFuntionGrp {

description "Represents the PCFFuntion IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at

least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a

Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based

interface.";

type inet:domain-name;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//optional support

reference "3GPP TS 23.003";

key "sd sst";

uses types5g3gpp:SNssai;

}

list managedNFProfile {

key idx;

min-elements 1;

max-elements 1;

uses types3gpp:ManagedNFProfile;

}

list commModelList {

min-elements 1;

key "groupId";

description "Specifies a list of commModel. It can be used by NF and

NF services to interact with each other in 5G Core network ";

reference "3GPP TS 23.501";

uses types5g3gpp:CommModel;

}

leaf dynamic5QISetRef {

type types3gpp:DistinguishedName;

description "DN of the Dynamic5QISet that the PCFFunction supports

(is associated to).";

}

leaf configurable5QISetRef {

type types3gpp:DistinguishedName;

description "DN of the Configurable5QISet that the PCFFunction supports

(is associated to).";

}

}

augment "/me3gpp:ManagedElement" {

list PCFFunction {

description "5G Core PCF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses PCFFuntionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## H.5.19	module _3gpp-5gc-nrm-seppfunction.yang

module _3gpp-5gc-nrm-seppfunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-seppfunction;

prefix sepp3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import ietf-inet-types { prefix inet; }

organization "3gpp SA5";

description "This IOC represents the SEPP function which support message filtering

and policing on inter-PLMN control plane interface. For more information about the SEPP, see 3GPP TS 23.501.";

reference "3GPP TS 28.541";

revision 2020-08-03 { reference "CR-0321"; }

revision 2019-10-28 { reference S5-193518 ; }

typedef SEPPType {

reference "3GPP TS 23501";

type enumeration {

enum CSEPP {

value 0;

description "consumer SEPP";

}

enum PSEPP {

value 1;

description "producer SEPP";

}

}

}

grouping SEPPFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

container pLMNId {

description "PLMN Identifiers of the sepp.

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

uses types3gpp:PLMNId;

}

leaf sEPPType {

type sepp3gpp:SEPPType;

}

leaf sEPPId {

type uint16;

}

leaf fqdn {

description "The domain name of the SEPP.";

type inet:domain-name;

}

}

augment "/me3gpp:ManagedElement" {

list SEPPFunction {

description "5G Core SEPP Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses SEPPFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;    }

}

}

## H.5.19a		module _3gpp-5gc-nrm- externalseppfunction@2019-11-17.yang

module _3gpp-5gc-nrm-externalseppfunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-extternalseppfunction;

prefix extsepp3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import ietf-inet-types { prefix inet; }

organization "3gpp SA5";

description "This IOC represents the external SEPP function which support message filtering

and policing on inter-PLMN control plane interface. For more information about the SEPP, see 3GPP TS 23.501.";

reference "3GPP TS 28.541";

revision 2019-11-17 {

description "initial revision";

reference "Based on

3GPP TS 28.541 V16.X.XX";

}

grouping ExternalSEPPFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

container pLMNId {

description "PLMN Identifiers of the sepp.

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

uses types3gpp:PLMNId;

}

leaf sEPPId {

type uint16;

}

leaf fqdn {

description "The domain name of the SEPP.";

type inet:domain-name;

}

}

augment "/me3gpp:ManagedElement" {

list ExternalSEPPFunction {

description "5G Core SEPP Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ExternalSEPPFunctionGrp;

}

}

}

}

## H.5.20	module _3gpp-5gc-nrm-smffunction.yang

module _3gpp-5gc-nrm-smffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-smffunction;

prefix smf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "SMFFunction derived from basic ManagedFunction.";

revision 2020-11-05 { reference CR-0412 ; }

revision 2020-08-06 { reference "CR-0333"; }

revision 2020-06-03 { reference "CR-0286"; }

revision 2019-10-25 { reference "S5-194457 S5-193518"; }

revision 2019-05-31 {reference "Ericsson refactoring."; }

revision 2018-08-07 { reference "Initial revision";}

grouping SMFFunctionGrp {

description "Represents the SMFFuntion IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

min-elements 1;

description "A list of PLMN identifiers (Mobile Country Code and Mobile

Network Code).";

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf-list nRTACList {

description "List of Tracking Area Codes (legacy TAC or extended TAC)

where the represented management function is serving.";

reference "TS 38.413 clause 9.3.3.10";

min-elements 1;

config false;

type types3gpp:Tac;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based

interface.";

type inet:domain-name;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

reference "3GPP TS 23.003";

key "sd sst";

uses types5g3gpp:SNssai;

}

list managedNFProfile {

key idx;

min-elements 1;

max-elements 1;

uses types3gpp:ManagedNFProfile;

}

list commModelList {

min-elements 1;

key "groupId";

description "Specifies a list of commModel. It can be used by NF and

NF services to interact with each other in 5G Core network ";

reference "3GPP TS 23.501";

uses types5g3gpp:CommModel;

}

leaf configurable5QISetRef {

type types3gpp:DistinguishedName;

description "DN of the Configurable5QISet that the SMFFunction supports

(is associated to).";

}

leaf dynamic5QISetRef {

type types3gpp:DistinguishedName;

description "DN of the Dynamic5QISet that the SMFFunction supports

(is associated to).";

}

}

augment "/me3gpp:ManagedElement" {

list SMFFunction {

description "5G Core SMF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses SMFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## H.5.21	module _3gpp-5gc-nrm-smsffunction@2019-10-25.yang

module _3gpp-5gc-nrm-smsffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-smsffunction;

prefix smsf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

description "This IOC represents the SMSF function defined in 3GPP TS 23.501.";

reference "3GPP TS 28.541";

revision 2019-10-25 { reference "S5-194457 S5-195427 S5-193518"; }

revision 2019-05-15 {

description "initial revision";

}

grouping SMSFFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

list managedNFProfile {

key idx;

min-elements 1;

uses types3gpp:ManagedNFProfile;

}

list commModelList {

min-elements 1;

key "groupId";

uses types5g3gpp:CommModel;

}

}

augment "/me3gpp:ManagedElement" {

list SMSFFunction {

description "5G Core SMSF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses SMSFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## H.5.22	module _3gpp-5gc-nrm-udmfunction.yang

module _3gpp-5gc-nrm-udmfunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-udmfunction;

prefix udm3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the UDM function in 5GC. For more

information about the UDM, see 3GPP TS 23.501.";

reference "3GPP TS 28.541";

revision 2020-11-05 { reference CR-0412 ; }

revision 2019-10-25 { reference "S5-194457 S5-195427 S5-193518"; }

revision 2019-05-22 { reference "initial revision";}

grouping UDMFuntionGrp {

description "Represents the UDMFuntion IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at

least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a

Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based

interface.";

type inet:domain-name;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//optional support

reference "3GPP TS 23.003";

key "sd sst";

uses types5g3gpp:SNssai;

}

list managedNFProfile {

key idx;

min-elements 1;

max-elements 1;

uses types3gpp:ManagedNFProfile;

}

list commModelList {

min-elements 1;

key "groupId";

description "Specifies a list of commModel. It can be used by NF and

NF services to interact with each other in 5G Core network ";

reference "3GPP TS 23.501";

uses types5g3gpp:CommModel;

}

}

augment "/me3gpp:ManagedElement" {

list UDMFunction {

description "5G Core UDM Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses UDMFuntionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## H.5.23	module _3gpp-5gc-nrm-udrfunction.yang

module _3gpp-5gc-nrm-udrfunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-udrfunction;

prefix udr3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the UDR function in 5GC. For more information

about the UDR, see 3GPP TS 23.501.";

reference "3GPP TS 28.541";

revision 2020-11-05 { reference CR-0412 ; }

revision 2019-10-25 { reference "S5-194457 S5-195427 S5-193518"; }

revision 2019-05-22 {reference "initial revision"; }

grouping UDRFuntionGrp {

description "Representse the UDRFuntion IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at

least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a

Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based

interface.";

type inet:domain-name;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//optional support

reference "3GPP TS 23.003";

key "sd sst";

uses types5g3gpp:SNssai;

}

list managedNFProfile {

key idx;

min-elements 1;

max-elements 1;

uses types3gpp:ManagedNFProfile;

}

}

augment "/me3gpp:ManagedElement" {

list UDRFunction {

description "5G Core UDR Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses UDRFuntionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## H.5.24	module _3gpp-5gc-nrm-udsffunction.yang

module _3gpp-5gc-nrm-udsffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-udsffunction;

prefix udsf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the UDSF function which can be interacted

with any other 5GC NF defined in 3GPP TS 23.501.";

reference "3GPP TS 28.541";

revision 2020-11-05 { reference CR-0412 ; }

revision 2019-10-25 { reference "S5-194457 S5-195427 S5-193518"; }

revision 2019-05-22 { reference "initial revision"; }

grouping UDSFFuntionGrp {

description "Represents the UDSFFuntion IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at

least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a

Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the

service-based interface.";

type inet:domain-name;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//optional support

reference "3GPP TS 23.003";

key "sd sst";

uses types5g3gpp:SNssai;

}

list managedNFProfile {

key idx;

min-elements 1;

max-elements 1;

description "Managed Network Function profile";

reference "3GPP TS 23.501";

uses types3gpp:ManagedNFProfile;

}

}

augment "/me3gpp:ManagedElement" {

list UDSFFunction {

description "5G Core UDSF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses UDSFFuntionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;    }

}

}

## H.5.25	module _3gpp-5gc-nrm-upffunction.yang

module _3gpp-5gc-nrm-upffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-upffunction;

prefix upf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "UPFFunction derived from basic ManagedFunction.";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2020-11-05 { reference CR-0412 ; }

revision 2019-10-25 { reference "S5-194457 S5-193518"; }

revision 2019-05-31 { reference "Ericsson refactoring."; }

revision 2018-08-07 { reference "Initial revision"; }

grouping UPFFunctionGrp {

description "Represents the UPFFunction IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "A list of PLMN identifiers (Mobile Country Code and Mobile

Network Code).";

min-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf-list nRTACList {

description "List of Tracking Area Codes (legacy TAC or extended TAC)

where the represented management function is serving.";

reference "TS 38.413 clause 9.3.3.10";

min-elements 1;

config false;

type types3gpp:Tac;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

reference "3GPP TS 23.003";

key "sd sst";

uses types5g3gpp:SNssai;

}

list managedNFProfile {

key idx;

min-elements 1;

max-elements 1;

reference "3GPP TS 23.003";

uses types3gpp:ManagedNFProfile;

}

leaf-list supportedBMOList {

type string;

description "List of supported BMOs (Bridge Managed Objects) required

for integration with TSN system.";

}

}

augment /me3gpp:ManagedElement {

list UPFFunction {

description "5G Core UPF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses UPFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## H.5.26	module _3gpp-5gc-nrm-scpfunction.yang

module _3gpp-5gc-nrm-scpfunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-scpfunction;

prefix scp3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

description "This IOC represents the SCP function in 5GC. For more information about the SCP, see 3GPP TS 23.501.";

reference "3GPP TS 28.541";

revision 2019-10-20 {

description "initial revision";

reference "Based on

3GPP TS 28.541 V16.X.XX";

}

grouping SCPFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

leaf address {

description "The host address of the SCP.";

type inet:host;

}

list supportedFuncList {

min-elements 1;

key "function";

uses types5g3gpp:SupportedFunc;

}

}

augment "/me3gpp:ManagedElement" {

list SCPFunction {

description "5G Core SCP Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses SCPFunctionGrp;

}

}

}

}

## H.5.27	module _3gpp-5gc-nrm-neffunction.yang

module _3gpp-5gc-nrm-neffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-neffunction;

prefix nef3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the NEF function in 5GC. For more

information about the NEF, see 3GPP TS 23.501.";

reference "3GPP TS 28.541";

revision 2020-11-05 { reference CR-0412 ; }

revision 2019-10-20 { reference "initial revision"; }

grouping NEFFunctionGrp {

description "Represents the NEFFunction IOC";

uses mf3gpp:ManagedFunctionGrp;

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the

service-based interface.";

type inet:domain-name;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

key "sd sst";

uses types5g3gpp:SNssai;

}

leaf-list capabilityList {

description "List of supported capabilities of the NEF.";

reference "3GPP TS 23.003";

type string;

}

leaf isINEF {

type boolean;

}

leaf isCAPIFSup {

type boolean;

}

}

augment "/me3gpp:ManagedElement" {

list NEFFunction {

description "5G Core NEF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NEFFunctionGrp;

}

}

}

}

## H.5.28	module _3gpp-5gc-nrm-QFQoSMonitoringControl.yang

module _3gpp-5gc-nrm-QFQoSMonitoringControl {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-QFQoSMonitoringControl;

prefix qFQMCtrl3gpp;

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-5gc-nrm-smffunction { prefix smf3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

organization "3gpp SA5";  contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the capabilities and properties for control

of QoS monitoring per QoS flow per UE for URLLC service defined

in 3GPP TS 23.501.";

reference "3GPP TS 28.541";

revision 2020-11-05 { reference CR-0412 ; }

revision 2020-08-03 { reference "CR-0321"; }

revision 2020-04-10 { reference "S5-202101";   }

grouping QFPacketDelayThresholdsTypeGrp {

description "Represents the QFPacketDelayThresholdsType";

leaf thresholdDl {

type uint32;

units milliseconds;

description "Downlink threshold";

}

leaf thresholdUl {

type uint32;

units milliseconds;

description "Uplink threshold";

}

leaf thresholdRtt {

type uint32;

units milliseconds;

description "Round trip threshold";

}

}

grouping QFQoSMonitoringControlGrp {

description "Represents the QFQoSMonitoringControl IOC.";

reference "3GPP TS 28.541";

leaf qFQoSMonitoringState {

description "The state of QoS monitoring per QoS flow per UE.";

mandatory true;

type enumeration {

enum ENABLED;

enum DISABLED;

}

}

list qFMonitoredSNSSAIs {

description "The S-NSSAIs for which the QoS monitoring per QoS flow

per UE is to be performed.";

reference "3GPP TS 23.003";

key "sd sst";

uses types5g3gpp:SNssai;}

leaf-list qFMonitored5QIs {

description "The 5QIs for which the QoS monitoring per QoS flow

per UE is to be performed.";

reference "3GPP TS 23.501";

type uint32 {

range "0..255";

}

}

leaf isEventTriggeredQFMonitoringSupported {

description "It indicates whether the event based QoS monitoring

reporting per QoS flow per UE is supported.";

mandatory true;

reference "3GPP TS 29.244";

type boolean;

}

leaf isPeriodicQFMonitoringSupported {

description "It indicates whether the periodic QoS monitoring reporting

per QoS flow per UE is supported.";

mandatory true;

reference "3GPP TS 29.244";

type boolean;

}

leaf isSessionReleasedQFMonitoringSupported {

description "It indicates whether the session release based QoS monitoring

reporting  per QoS flow per UE is supported.";

mandatory true;

reference "3GPP TS 29.244";

type boolean;

}

list qFPacketDelayThresholds {

key "idx";

min-elements 1;

max-elements 1;

description "It specifies the thresholds for reporting the packet delay

between PSA and UE for QoS monitoring per QoS flow per UE.";

leaf idx { type uint32 ; }

uses QFPacketDelayThresholdsTypeGrp;

}

leaf qFMinimumWaitTime {

description "It specifies the minimum waiting time (in seconds) between

two consecutive reports for event triggered QoS monitoring reporting

per QoS flow per UE.";

type uint32;

}

leaf qFMeasurementPeriod {

description "It specifies the period (in seconds) for reporting the

packet delay for QoS monitoring per QoS flow per UE.";

type uint32;

}

}

augment "/me3gpp:ManagedElement/smf3gpp:SMFFunction" {

list QFQoSMonitoringControl {

description "Represents the QFQoSMonitoringControl IOC.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses QFQoSMonitoringControlGrp;

}

}

}

}

## H.5.29	module _3gpp-5gc-nrm-GtpUPathQoSMonitoringControl.yang

module _3gpp-5gc-nrm-GtpUPathQoSMonitoringControl {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-GtpUPathQoSMonitoringControl;

prefix gtpUPathQMCtrl3gpp;

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-5gc-nrm-smffunction { prefix smf3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the capabilities and properties for control

of GTP-U path QoS monitoring defined in 3GPP TS 23.501.";

reference "3GPP TS 28.541";

revision 2020-11-05 { reference CR-0412 ; }

revision 2020-09-30 { reference "CR-0393"; }

revision 2020-08-03 { reference "CR-0321"; }

revision 2020-04-10 { reference "S5-202103"; }

grouping GtpUPathDelayThresholdsType {

description "Thresholds for reporting the packet delay for GTP-U path QoS

monitoring ";

reference "3GPP TS 29.244";

leaf n3AveragePacketDelayThreshold {

mandatory true;

type uint32;

}

leaf n3MinPacketDelayThreshold {

mandatory true;

type uint32;

}

leaf n3MaxPacketDelayThreshold {

mandatory true;

type uint32;

}

leaf n9AveragePacketDelayThreshold {

mandatory true;

type uint32;

}

leaf n9MinPacketDelayThreshold {

mandatory true;

type uint32;

}

leaf n9MaxPacketDelayThreshold {

mandatory true;

type uint32;

}

}

grouping GtpUPathQoSMonitoringControlGrp {

description "Represents the GtpUPathQoSMonitoringControl IOC.";

leaf gtpUPathQoSMonitoringState {

description "The state of GTP-U path QoS monitoring.";

mandatory true;

type enumeration {

enum ENABLED;

enum DISABLED;

}

}

list gtpUPathMonitoredSNSSAIs {

key "sd sst";

description "The S-NSSAIs for which the the GTP-U path QoS monitoring is

to be performed.";

reference "3GPP TS 23.003";

uses types5g3gpp:SNssai;

}

leaf-list monitoredDSCPs {

description "The DSCPs for which the GTP-U path QoS monitoring is to be

performed.";

reference "3GPP TS 29.244";

type uint32;

}

leaf isEventTriggeredGtpUPathMonitoringSupported {

description "It indicates whether the event triggered GTP-U path QoS

monitoring reporting based on thresholds is supported.";

mandatory true;

reference "3GPP TS 29.244";

type boolean;

}

leaf isPeriodicGtpUMonitoringSupported {

description "It indicates whether the periodic GTP-U path QoS monitoring

reporting is supported.";

mandatory true;

reference "3GPP TS 29.244";

type boolean;

}

leaf isImmediateGtpUMonitoringSupported {

description "It indicates whether the immediate GTP-U path QoS monitoring

reporting is supported.";

mandatory true;

reference "3GPP TS 29.244";

type boolean;

}

list gtpUPathDelayThresholds {

key n3AveragePacketDelayThreshold;

// if max-elements is increased later, the key may need to be modified

min-elements 1;

max-elements 1;

description "It specifies the thresholds for reporting the packet delay

for the GTO-U path QoS monitoring.";

uses GtpUPathDelayThresholdsType;

}

leaf gtpUPathMinimumWaitTime {

description "It specifies the minimum waiting time (in seconds) between

two consecutive reports for event triggered GTP-U path QoS monitoring

reporting.";

type uint32;

}

leaf gtpUPathMeasurementPeriod {

description "It specifies the period (in seconds) for reporting the packet

delay for GTP-U path QoS monitoring.";

type uint32;

}

}

augment "/me3gpp:ManagedElement/smf3gpp:SMFFunction" {

list GtpUPathQoSMonitoringControl {

description "Specifies the capabilities and properties for control of

GTP-U path QoS monitoring. For more information about the GTP-U path

QoS monitoring.";

reference "3GPP TS 23.501";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses GtpUPathQoSMonitoringControlGrp;

}

}

}

}

## H.5.30	module _3gpp-5gc-nrm-Configurable5QISet.yang

module _3gpp-5gc-nrm-configurable5qiset {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-configurable5qiset;

prefix Conf5QIs3gpp;

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the non-standardized 5QIs, including

their QoS characteristics, that need to be pre-configured

(and configurable) to the 5G NFs.";

reference "3GPP TS 28.541";

revision 2020-08-03 { reference "CR-0321"; }

revision 2020-06-03 { reference "CR-0286"; }

grouping PacketErrorRate {

leaf scalar {

type uint32 {

range 0..9 ;

}

mandatory true;

description "The Packet Error Rate of a 5QI expressed as Scalar x 10-k

where k is the Exponent.";

}

leaf exponent {

type uint32 {

range 0..9 ;

}

mandatory true;

description "The Packet Error Rate of a 5QI expressed as Scalar x 10-k,

where k is the Exponent.";

}

}

grouping FiveQICharacteristics {

leaf fiveQIValue {

type uint32 {

range 0..255 ;

}

mandatory true;

description "Identifies the 5QI value.";

}

leaf resourceType {

type enumeration {

enum GBR;

enum NON_GBR;

}

mandatory true;

description "It indicates the Resource Type of a 5QI, as specified

in TS 23.501 ";

}

leaf priorityLevel {

type uint32 {

range 0..127 ;

}

}

leaf packetDelayBudget {

type uint32 {

range 0..1023 ;

}

description "Indicates the Packet Delay Budget (in unit of 0.5ms)of a 5QI,

as specified in TS 23.501 ";

}

list packetErrorRate {

key "scalar exponent";

min-elements 0;

max-elements 1;

uses PacketErrorRate;

reference "TS 23.501";

}

leaf averagingWindow {

type uint32 {

range 0..4095 ;

}

units ms;

reference "TS 23.501";

}

leaf maximumDataBurstVolume {

type uint32{

range 0..4095 ;

}

units byte;

}

}

grouping Configurable5QISetGrp {

description "Represents the Configurable5QISet IOC.";

list configurable5QIs {

key "fiveQIValue";

uses FiveQICharacteristics;

}

}

grouping Configurable5QISetSubtree {

list Configurable5QISet {

description "Specifies the non-standardized 5QIs, including their QoS

characteristics, that need to be pre-configured (and configurable) to

the 5G NFs, see 3GPP TS 23.501.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses Configurable5QISetGrp;

}

}

}

augment "/subnet3gpp:SubNetwork" {

uses Configurable5QISetSubtree;

}

augment "/me3gpp:ManagedElement" {

uses Configurable5QISetSubtree;

}

}

## H.5.31	module _3gpp-5gc-nrm-FiveQiDscpMappingSet.yang

module _3gpp-5gc-nrm-FiveQiDscpMappingSet {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-FiveQiDscpMappingSet;

prefix FiveQiDscpMapping3gpp;

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-5gc-nrm-smffunction { prefix smf3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description " This IOC represents the set of mapping between 5QIs and DSCP.";

reference "3GPP TS 28.541";

revision 2020-08-03 { reference "CR-0321"; }

revision 2020-05-27 { reference "CR-0287"; }

grouping FiveQiDscpMapping {

leaf-list fiveQIValues {

type uint32 {

range 0..255 ;

}

min-elements 1;

description " Identifies the 5QI values that are mapped to a same DSCP, as specified in TS 28.541.";

}

leaf dscp {

type uint32 {

range 0..255 ;

}

mandatory true;

}

}

grouping FiveQiDscpMappingSetGrp {

description "Represents the FiveQiDscpMappingSet IOC.";

list FiveQiDscpMappingList {

key "dscp";

uses FiveQiDscpMapping;

}

}

grouping FiveQiDscpMappingSetSubtree {

list FiveQiDscpMappingSet {

description "Specifies the mapping between 5QIs and DSCPs.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses FiveQiDscpMappingSetGrp;

}

}

}

augment "/me3gpp:ManagedElement/smf3gpp:SMFFunction" {

uses FiveQiDscpMappingSetSubtree;

}

}

## H.5.32	module _3gpp-5gc-nrm-PredefinedPccRuleSet.yang

module _3gpp-5gc-nrm-predefinedpccruleset {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-predefinedpccruleset;

prefix PredPccRules3gpp;

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-5gc-nrm-smffunction { prefix smf3gpp; }

import _3gpp-5gc-nrm-pcffunction { prefix pcf3gpp; }

import ietf-yang-types { prefix yang; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the predefined PCC rules, which are

configured to SMF and referenced by PCF.";

reference "3GPP TS 28.541";

revision 2020-09-30 { reference "CR-0393"; }

revision 2020-08-21 { reference "CR-0330"; }

grouping TscaiInputContainer {

description "It specifies the transports TSCAI input parameters for TSC

traffic at the ingress interface of the DS-TT/UE for a PCC rule.";

reference " 3GPP TS 29.512";

leaf periodicity {

type uint32;

description "It identifies the time period between the start of two bursts

in reference to the TSN GM.";

reference "3GPP TS 29.571.";

}

leaf burstArrivalTime {

type yang:date-and-time;

description "It Indicates the arrival time (in date-time format) of the

data burst in reference to the TSN GM.";

reference "3GPP,TS 29.571.";

}

}

grouping ConditionData {

description "It specifies the specifies the condition data for a PCC rule.";

leaf condId {

type string;

mandatory true;

description "It uniquely identifies the condition data.";

}

leaf activationTime {

type yang:date-and-time;

description " It indicates the time (in date-time format) when the decision

data shall be activated.";

reference "3GPP29.512 and TS 29.571.";

}

leaf deactivationTime {

type yang:date-and-time;

description "It indicates the time (in date-time format) when the decision

data shall be deactivated.";

reference "3GPPTS 29.512 and TS 29.571.";

}

leaf accessType {

type enumeration {

enum 3GPP_ACCESS;

enum NON_3GPP_ACCESS;

}

description "It provides the condition of access type of the UE when the

session AMBR shall be enforced.";

reference "3GPPTS 29.512.";

}

leaf ratType {

type enumeration {

enum NR;

enum EUTRA;

enum WLAN;

enum VIRTUAL;

enum NBIOT;

enum WIRELINE;

enum WIRELINE_CABLE;

enum WIRELINE_BBF;

enum LTE-M;

enum NR_U;

enum EUTRA_U;

enum TRUSTED_N3GA;

enum TRUSTED_WLAN;

enum UTRA;

enum GERA;

}

description "It provides the condition of RAT type of the UE when the

session AMBR shall be enforced.";

reference "3GPPTS 29.512 and TS 29.571.";

}

}

grouping SteeringMode {

description "It specifies the traffic distribution rule, see TS 29.512.";

leaf steerModeValue {

type enumeration {

enum ACTIVE_STANDBY;

enum LOAD_BALANCING;

enum SMALLEST_DELAY;

enum PRIORITY_BASED;

}

mandatory true;

description "It indicates the value of the steering mode, see TS 29.512.";

}

leaf active {

type enumeration {

enum 3GPP_ACCESS;

enum NON_3GPP_ACCESS;

}

description "It indicates the active access, see TS 29.571.";

}

leaf standby {

type enumeration {

enum 3GPP_ACCESS;

enum NON_3GPP_ACCESS;

}

description "It indicates the Standby access, see TS 29.571.";

}

leaf threeGLoad {

type uint8 {

range 0..100;

}

description "It indicates the traffic load to steer to the 3GPP Access

expressed in one percent.";

}

leaf prioAcc {

type enumeration {

enum 3GPP_ACCESS;

enum NON_3GPP_ACCESS;

}

description "It indicates the high priority access.";

reference "3GPPTS 29.571";

}

}

grouping UpPathChgEvent {

description "It specifies the information about the AF subscriptions of the

UP path change.";

reference "3GPPTS 29.512";

leaf notificationUri {

type string;

mandatory true;

description "It provides notification address (Uri) of AF receiving the

event notification.";

}

leaf notifCorreId {

type string;

mandatory true;

description "It is used to set the value of Notification Correlation ID in

the notification sent by the SMF, see TS 29.512";

}

leaf dnaiChgType {

type enumeration {

enum EARLY;

enum EARLY_LATE;

enum LATE;

}

mandatory true;

description "It indicates the type of DNAI change, see TS 29.512";

}

leaf afAckInd {

type boolean;

default false;

description "It identifies whether the AF acknowledgement of UP path

event notification is expected.";

}

}

grouping RouteInformation {

description "It specifies the traffic routing information.";

leaf ipv4Addr {

type string;

description "It defines the Ipv4 address of the tunnel end point in the

data network, formatted in the dotted decimal notation.";

}

leaf ipv6Addr {

type string;

description "It defines the Ipv6 address of the tunnel end point in the

data network.";

}

leaf portNumber {

type uint32;

mandatory true;

description " It defines the UDP port number of the tunnel end point in

the data network, see TS 29.571";

}

}

grouping RouteToLocation {

description "It specifies a list of location which the traffic shall be

routed to for the AF request.";

leaf dnai {

type string;

mandatory true;

description "It represents the DNAI (Data network access identifier).";

reference "3GPP 3GPP TS 23.501";

}

container routeInfo{

description "It provides the traffic routing information.";

uses RouteInformation;

}

leaf routeProfId {

type string;

description "It identifies the routing profile.";

}

}

grouping RedirectInformaton {

description "It specifies the redirect information for traffic control in

the PCC rule.";

leaf redirectEnabled {

type boolean;

mandatory true;

description "It indicates whether the redirect instruction is enabled.";

}

leaf redirectAddressType {

type enumeration {

enum IPV4_ADDR;

enum IPV6_ADDR;

enum URL;

enum SIP_URI;

}

mandatory true;

description "It indicates the type of redirect address.";

reference "3GPPTS 29.512";

}

leaf redirectServerAddress {

type string;

mandatory true;

description "It indicates the address of the redirect server.";

}

}

grouping TrafficControlDataInformation {

description "It specifies the traffic control data for a service

flow of a PCC rule.";

leaf tcId {

type string;

mandatory true;

description "It univocally identifies the traffic control policy data

within a PDU session.";

}

leaf flowStatus {

type enumeration {

enum ENABLED-UPLINK;

enum ENABLED-DOWNLINK;

enum ENABLED;

enum DISABLED;

enum REMOVED;

}

mandatory true;

description "It represents whether the service data flow(s) are enabled

or disabled.";

}

container redirectInfo {

description "It contains the redirect information indicating whether the

detected application traffic should be redirected to another controlled

address.";

uses RedirectInformaton;

}

container addRedirectInfo {

description "It contains the additional redirect information indicating

whether the detected application traffic should be redirected to another

controlled address.";

list redirectInfo {

description "The list of redirect information indicating whether the

detected application traffic should be redirected to another

controlled address.";

key "redirectServerAddress";

uses RedirectInformaton;

}

}

leaf muteNotif {

type boolean;

default false;

description "It indicates whether applicat'on's start or stop notification

is to be muted.";

}

leaf trafficSteeringPolIdDl {

type string;

description "It references to a pre-configured traffic steering policy for downlink traffic at the SMF, see TS 29.512";

}

leaf trafficSteeringPolIdUl {

type string;

mandatory false;

description "It references to a pre-configured traffic steering policy for

uplink traffic at the SMF, see TS 29.512";

}

container routeToLocs {

description "It provides a list of location which the traffic shall be

routed to for the AF request.";

list routeToLoc {

description "The list of location which the traffic shall be routed to

for the AF request.";

key "dnai";

uses RouteToLocation;

}

}

uses UpPathChgEvent;

leaf steerFun {

type enumeration {

enum MPTCP;

enum ATSSS_LL;

}

description "It indicates the applicable traffic steering functionality.";

reference "3GPPTS 29.512";

}

container steerModeDl {

description "It provides the traffic distribution rule across 3GPP and

Non-3GPP accesses to apply for downlink traffic.";

uses SteeringMode;

}

container steerModeUl {

description "It provides the traffic distribution rule across 3GPP and Non-3GPP accesses to apply for uplink traffic.";

uses SteeringMode;

}

leaf mulAccCtrl {

type enumeration {

enum ALLOWED;

enum NOT_ALLOWED;

}

description "It indicates whether the service data flow, corresponding to the service data flow template, is allowed or not allowed.";

}

}

grouping ARP {

description "It specifies the allocation and retention priority of a QoS control policy.";

leaf priorityLevel {

type uint8 {

range 1..15;

}

mandatory true;

description "It defines the relative importance of a resource request.";

}

leaf preemptCap {

type enumeration {

enum NOT_PREEMPT;

enum MAY_PREEMPT;

}

mandatory true;

description "It defines whether a service data flow may get resources that were already assigned to another service data flow with a lower priority level.";

}

leaf preemptVuln {

type enumeration {

enum NOT_PREEMPTABLE;

enum PREEMPTABLE;

}

mandatory true;

description "It defines whether a service data flow may lose the resources assigned to it in order to admit a service data flow with higher priority level.";

}

}

grouping QosDataInformation {

description "It specifies the QoS control policy data for a service flow of a PCC rule.";

leaf qosId {

type string;

mandatory true;

description "It identifies the QoS control policy data for a PCC rule.";

}

leaf fiveQIValue {

type uint8 {

range 0..255;

}

mandatory true;

description "It indicates the 5QI value.";

}

leaf maxbrUl {

type string;

description "It represents the maximum uplink bandwidth.";

}

leaf maxbrDl {

type string;

description "It represents the maximum downlink bandwidth.";

}

leaf gbrUl {

type string;

description "It represents the guaranteed uplink bandwidth.";

}

leaf gbrDl {

type string;

description "It represents the guaranteed downlink bandwidth.";

}

uses ARP;

leaf qosNotificationControl {

type boolean;

default false;

description "It indicates whether notifications are requested from 3GPP NG-RAN when the GFBR can no longer (or again) be guaranteed for a QoS Flow during the lifetime of the QoS Flow.";

}

leaf reflectiveQos {

type boolean;

default false;

description "Indicates whether the QoS information is reflective for the corresponding non-GBR service data flow";

}

leaf sharingKeyDl {

type string;

description "It indicates, by containing the same value, what PCC rules may share resource in downlink direction.";

}

leaf sharingKeyUl {

type string;

description "It indicates, by containing the same value, what PCC rules may share resource in uplink direction.";

}

leaf maxPacketLossRateDl {

type uint16 {

range 0..1000;

}

description "It indicates the downlink maximum rate for lost packets that can be tolerated for the service data flow.";

}

leaf maxPacketLossRateUl {

type uint16 {

range 0..1000;

}

description "It indicates the uplink maximum rate for lost packets that can be tolerated for the service data flow.";

}

leaf extMaxDataBurstVol {

type uint32 {

range 4096..2000000;

}

description "It denotes the largest amount of data that is required to be transferred within a period of 5G-AN PDB, see TS 29.512";

}

}

grouping EthFlowDescription {

description "It describes an Ethernet flow.";

leaf destMacAddr {

type string;

mandatory true;

description "It specifies the destination MAC address formatted in the

hexadecimal .";

reference "clause 1.1 and clause 2.1 of IETF RFC";

}

leaf ethType {

type string;

mandatory true;

description "A two-octet string that represents the Ethertype.";

reference "IEEE 802.3 and IETF RFC 7042 in hexadecimal representation.";

}

leaf fDesc {

type string;

description "It contains the flow description for the Uplink or Downlink IP flow. It shall be present when the ethtype is IP.";

}

leaf fDir {

type enumeration {

enum DOWNLINK;

enum UPLINK;

}

mandatory true;

description "It indicates the packet filter direction.";

}

leaf sourceMacAddr {

type string;

mandatory true;

description "It specifies the source MAC address formatted in the

hexadecimal notation.";

reference "clause 1.1 and clause 2.1 of IETF RFC 7042";

}

leaf-list vlanTags {

type string;

description "It specifies the Customer-VLAN and/or Service-VLAN tags

containing the VID, PCP/DEI fields as defined in IEEE 802.1Q and

IETF RFC 7042. The first/lower instance in the array stands for the

Customer-VLAN tag and the second/higher instance in the array stands

for the Service-VLAN tag.";

}

leaf srcMacAddrEnd {

type string;

description "It specifies the source MAC address end. If this attribute is present, the sourceMacAddr attribute specifies the source MAC address start. E.g. srcMacAddrEnd with value 00-10-A4-23-3E-FE and sourceMacAddr with value 00-10-A4-23-3E-02 means all MAC addresses from 00-10-A4-23-3E-02 up to and including 00-10-A4-23-3E-FE.";

}

leaf destMacAddrEnd {

type string;

description "It specifies the destination MAC address end. If this attribute is present, the destMacAddr attribute specifies the destination MAC address start.";

}

}

grouping FlowInformation {

description "It specifies the flow information of a PCC rule.";

leaf flowDescription {

type string;

mandatory true;

description "It defines a packet filter for an IP flow.";

}

uses EthFlowDescription;

leaf packFiltId {

type string;

mandatory true;

description "It is the identifier of the packet filter.";

}

leaf packetFilterUsage {

type boolean;

default false;

description "It indicates if the packet shall be sent to the UE.";

}

leaf tosTrafficClass {

type string;

mandatory true;

description "It contains the Ipv4 Type-of-Service and mask field or the Ipv6 Traffic-Class field and mask field.";

}

leaf spi {

type string;

mandatory true;

description "It is the security parameter index of the IPSec packet.";

reference "IETF RFC 4301";

}

leaf flowLabel {

type string;

description "It specifies the Ipv6 flow label header field.";

}

leaf flowDirection {

type enumeration {

enum DOWNLINK;

enum UPLINK;

enum BIDIRECTIONAL;

enum UNSPECIFIED;

}

mandatory true;

description "It indicates the direction/directions that a filter is applicable.";

}

}

grouping PccRule {

description "It specifies the PCC rule, see TS 29.512";

leaf pccRuleId {

type string;

mandatory true;

description "It identifies the PCC rule.";

}

container flowInfoList {

description "It is a list of IP flow packet filter information.";

list flowInfo {

description "The list of IP flow packet filter information.";

key "packFiltId";

uses FlowInformation;

}

}

leaf applicationId {

type string;

default false;

description "A reference to the application detection filter configured

at the UPF.";

}

leaf appDescriptor {

type string;

description "It is the ATSSS rule application descriptor.";

}

leaf contentVersion {

type uint8;

description "Indicates the content version of the PCC rule.";

}

leaf precedence {

type uint8 {

range 0..255;

}

description "It indicates the order in which this PCC rule is applied

relative to other PCC rules within the same PDU session.";

}

leaf afSigProtocol {

type enumeration {

enum NO_INFORMATION;

enum SIP;

}

description "Indicates the protocol used for signalling between the UE

and the AF, the default value is NO_INFORMATION.";

}

leaf isAppRelocatable {

type boolean;

default false;

mandatory false;

description "It indicates the application relocation possibility, the

default value is NO_INFORMATION.";

}

leaf isUeAddrPreserved {

type boolean;

default false;

description "It Indicates whether UE IP address should be preserved.";

}

container qosData {

description "It contains the QoS control policy data for a PCC rule.";

list qosDataInfo {

description "The list of QoS control policy data.";

key "qosId";

uses QosDataInformation;

}

}

container altQosParams {

description "It contains the QoS control policy data for the Alternative

QoS parameter sets of the service data flow.";

list qosDataInfo {

description "The list of QoS control policy data.";

key "qosId";

uses QosDataInformation;

}

}

container trafficControlData {

description "It contains the traffic control policy data for a PCC rule.";

list trafficControlDataInfo {

description "The list of traffic control policy data.";

key "tcId";

uses TrafficControlDataInformation;

}

}

uses ConditionData;

container tscaiInputUl {

description "It contains transports TSCAI input parameters for TSC traffic at the ingress interface of the DS-TT/UE (uplink flow direction).";

uses TscaiInputContainer;

}

container tscaiInputDl {

description "It contains transports TSCAI input parameters for TSC traffic at the ingress of the NW-TT (downlink flow direction).";

uses TscaiInputContainer;

}

}

grouping PredefinedPccRuleSetGrp {

description "Represents the PredefinedPccRuleSet IOC.";

list PredefinedPccRules {

description "The list of predefined PCC rules.";

key "pccRuleId";

uses PccRule;

}

}

grouping PredefinedPccRuleSetSubtree {

description "It specifies the PredefinedPccRuleSet IOC with inherited attributes.";

list PredefinedPccRuleSet {

description "Specifies the predefined PCC rules.";

key "id";

uses top3gpp:Top_Grp;

container attributes {

description "It contains the attributes defined specifically in the PredefinedPccRuleSet IOC.";

uses PredefinedPccRuleSetGrp;

}

}

}

augment "/me3gpp:ManagedElement/smf3gpp:SMFFunction" {

description "It specifies the containment relation of PredefinedPccRuleSet MOI with SMFFunction MOI.";

uses PredefinedPccRuleSetSubtree;

}

augment "/me3gpp:ManagedElement/pcf3gpp:PCFFunction" {

description "It specifies the containment relation of PredefinedPccRuleSet MOI with PCFFunction MOI.";

uses PredefinedPccRuleSetSubtree;

}

}

## H.5.33	module _3gpp-5gc-nrm-dynamic5QISet@2020-08-06.yang

module _3gpp-5gc-nrm-dynamic5qiset {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-dynamic5qiset;

prefix dyn5QIs3gpp;

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-5gc-nrm-configurable5qiset { prefix Conf5QIs3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the dynamic 5QIs including their QoS

characteristics.";

reference "3GPP TS 28.541";

revision 2020-09-30 { reference "CR-0393"; }

revision 2020-08-06 { reference "CR-0333"; }

grouping Dynamic5QISetGrp {

description "Represents the Dynamic5QISet IOC.";

list dynamic5QIs {

key "fiveQIValue";

description "Represents the Dynamic5QISet IOC.";

uses Conf5QIs3gpp:FiveQICharacteristics;

}

}

grouping Dynamic5QISetSubtree {

description "Helps augmenting Dynamic5QISet into multiple places.";

list Dynamic5QISet {

description "Specifies the dynamic 5QIs including their QoS

characteristics, see 3GPP TS 23.501.";

key "id";

uses top3gpp:Top_Grp;

container attributes {

uses Dynamic5QISetGrp;

}

}

}

augment "/subnet3gpp:SubNetwork" {

uses Dynamic5QISetSubtree;

}

augment "/me3gpp:ManagedElement" {

uses Dynamic5QISetSubtree;

}

}

# H.6	Void

# H.7	Mount information

_3gpp-5gc-nrm-affunction.yang

_3gpp-5gc-nrm-amffunction.yang

_3gpp-5gc-nrm-amfregion.yang

_3gpp-5gc-nrm-amfset.yang

_3gpp-5gc-nrm-ausffunction.yang

_3gpp-5gc-nrm-dnfunction.yang

_3gpp-5gc-nrm-ep.yang

_3gpp-5gc-nrm-externalnrffunction.yang

_3gpp-5gc-nrm-externalnssffunction.yang

_3gpp-5gc-nrm-lmffunction.yang

_3gpp-5gc-nrm-n3iwffunction.yang

_3gpp-5gc-nrm-nfprofile.yang

_3gpp-5gc-nrm-nfservice.yang

_3gpp-5gc-nrm-ngeirfunction.yang

_3gpp-5gc-nrm-nrffunction.yang

_3gpp-5gc-nrm-nssffunction.yang

_3gpp-5gc-nrm-nwdaffunction.yang

_3gpp-5gc-nrm-pcffunction.yang

_3gpp-5gc-nrm-seppfunction.yang

_3gpp-5gc-nrm-smffunction.yang

_3gpp-5gc-nrm-smsffunction.yang

_3gpp-5gc-nrm-udmfunction.yang

_3gpp-5gc-nrm-udrfunction.yang

_3gpp-5gc-nrm-udsffunction.yang

_3gpp-5gc-nrm-upffunction.yang

If the above files are mounted the yang files described in clause E.7 shall also be mounted.

Annex I (normative):
Void

Annex J (normative):
OpenAPI definition of the Slice NRM

# J.1	General

This annex contains the OpenAPI definition of the Slice NRM in YAML format.

The Information Service (IS) of the NR NRM is defined in clause 6.

Mapping rules to produce the OpenAPI definition based on the IS are defined in 3GPP TS 32.160 [14].

# J.2	Void

# J.3	Void

# J.4	Solution Set (SS) definitions

## J.4.1	Void

## J.4.2	Void

## J.4.3	OpenAPI document "sliceNrm.yaml"

openapi: 3.0.1

info:

title: Slice NRM

version: 17.2.0

description: >-

OAS 3.0.1 specification of the Slice NRM

@ 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.541 V17.2.0; 5G NRM, Slice NRM

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.541/

paths: {}

components:

schemas:

#------------ Type definitions ---------------------------------------------------

Float:

type: number

format: float

MobilityLevel:

type: string

enum:

- STATIONARY

- NOMADIC

- RESTRICTED MOBILITY

- FULLY MOBILITY

SynAvailability:

type: string

enum:

- NOT SUPPORTED

- BETWEEN BS AND UE

- BETWEEN BS AND UE & UE AND UE

PositioningAvailability:

type: array

items:

type: string

enum:

- CIDE-CID

- OTDOA

- RF FINGERPRINTING

- AECID

- HYBRID POSITIONING

- NET-RTK

Predictionfrequency:

type: string

enum:

- PERSEC

- PERMIN

- PERHOUR

SharingLevel:

type: string

enum:

- SHARED

- NON-SHARED

NetworkSliceSharingIndicator:

type: string

enum:

- SHARED

- NON-SHARED

ServiceType:

type: string

enum:

- eMBB

- RLLC

- MIoT

- V2X

SliceSimultaneousUse:

type: string

enum:

- ZERO

- ONE

- TWO

- THREE

- FOUR

PerfReqEmbb:

type: object

properties:

expDataRateDL:

type: number

expDataRateUL:

type: number

areaTrafficCapDL:

type: number

areaTrafficCapUL:

type: number

userDensity:

type: number

activityFactor:

type: number

PerfReqEmbbList:

type: array

items:

$ref: '#/components/schemas/PerfReqEmbb'

PerfReqUrllc:

type: object

properties:

cSAvailabilityTarget:

type: number

cSReliabilityMeanTime:

type: string

expDataRate:

type: number

msgSizeByte:

type: string

transferIntervalTarget:

type: string

survivalTime:

type: string

PerfReqUrllcList:

type: array

items:

$ref: '#/components/schemas/PerfReqUrllc'

PerfReq:

oneOf:

- $ref: '#/components/schemas/PerfReqEmbbList'

- $ref: '#/components/schemas/PerfReqUrllcList'

Category:

type: string

enum:

- CHARACTER

- SCALABILITY

Tagging:

type: array

items:

type: string

enum:

- PERFORMANCE

- FUNCTION

- OPERATION

Exposure:

type: string

enum:

- API

- KPI

ServAttrCom:

type: object

properties:

category:

$ref: '#/components/schemas/Category'

tagging:

$ref: '#/components/schemas/Tagging'

exposure:

$ref: '#/components/schemas/Exposure'

Support:

type: string

enum:

- NOT SUPPORTED

- SUPPORTED

DelayTolerance:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

support:

$ref: '#/components/schemas/Support'

DeterministicComm:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

availability:

$ref: '#/components/schemas/Support'

periodicityList:

type: string

DLThptPerSlice:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

guaThpt:

$ref: '#/components/schemas/Float'

maxThpt:

$ref: '#/components/schemas/Float'

DLThptPerUE:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

guaThpt:

$ref: '#/components/schemas/Float'

maxThpt:

$ref: '#/components/schemas/Float'

ULThptPerSlice:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

guaThpt:

$ref: '#/components/schemas/Float'

maxThpt:

$ref: '#/components/schemas/Float'

ULThptPerUE:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

guaThpt:

$ref: '#/components/schemas/Float'

maxThpt:

$ref: '#/components/schemas/Float'

DLThptPerSliceSubnet:

type: object

properties:

guaThpt:

$ref: '#/components/schemas/Float'

maxThpt:

$ref: '#/components/schemas/Float'

DLThptPerUEPerSubnet:

type: object

properties:

guaThpt:

$ref: '#/components/schemas/Float'

maxThpt:

$ref: '#/components/schemas/Float'

ULThptPerSliceSubnet:

type: object

properties:

guaThpt:

$ref: '#/components/schemas/Float'

maxThpt:

$ref: '#/components/schemas/Float'

ULThptPerUEPerSubnet:

type: object

properties:

guaThpt:

$ref: '#/components/schemas/Float'

maxThpt:

$ref: '#/components/schemas/Float'

MaxPktSize:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

maxsize:

type: integer

MaxNumberofPDUSessions:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

nOofPDUSessions:

type: integer

KPIMonitoring:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

kPIList:

type: string

NBIoT:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

support:

$ref: '#/components/schemas/Support'

Synchronicity:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

availability:

$ref: '#/components/schemas/SynAvailability'

accuracy:

$ref: '#/components/schemas/Float'

Positioning:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

availability:

$ref: '#/components/schemas/PositioningAvailability'

predictionfrequency:

$ref: '#/components/schemas/Predictionfrequency'

accuracy:

$ref: '#/components/schemas/Float'

UserMgmtOpen:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

support:

$ref: '#/components/schemas/Support'

V2XCommModels:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

v2XMode:

$ref: '#/components/schemas/Support'

TermDensity:

type: object

properties:

servAttrCom:

$ref: '#/components/schemas/ServAttrCom'

density:

type: integer

NsInfo:

type: object

properties:

nsInstanceId:

type: string

nsName:

type: string

CNSliceSubnetProfile:

type: object

properties:

maxNumberofUEs:

type: integer

latency:

type: integer

dLThptPerSliceSubnet:

$ref: '#/components/schemas/DLThptPerSliceSubnet'

dLThptPerUEPerSubnet:

$ref: '#/components/schemas/DLThptPerUEPerSubnet'

uLThptPerSliceSubnet:

$ref: '#/components/schemas/ULThptPerSliceSubnet'

uLThptPerUEPerSubnet:

$ref: '#/components/schemas/ULThptPerUEPerSubnet'

maxNumberOfPDUSessions:

type: integer

maxPktSize:

type: integer

delayTolerance:

$ref: '#/components/schemas/DelayTolerance'

sliceSimultaneousUse:

$ref: '#/components/schemas/SliceSimultaneousUse'

RANSliceSubnetProfile:

type: object

properties:

coverageAreaTAList:

type: integer

MobilityLevel:

$ref: '#/components/schemas/MobilityLevel'

resourceSharingLevel:

$ref: '#/components/schemas/SharingLevel'

maxNumberofUEs:

type: integer

activityFactor:

type: integer

dLThptPerUEPerSubnet:

$ref: '#/components/schemas/ULThptPerUEPerSubnet'

uLThptPerUEPerSubnet:

$ref: '#/components/schemas/ULThptPerUEPerSubnet'

uESpeed:

type: integer

reliability:

type: string

serviceType:

$ref: '#/components/schemas/ServiceType'

maxPktSize:

type: integer

delayTolerance:

$ref: '#/components/schemas/DelayTolerance'

sliceSimultaneousUse:

$ref: '#/components/schemas/SliceSimultaneousUse'

TopSliceSubnetProfile:

type: object

properties:

coverageArea:

type: string

latency:

type: integer

maxNumberofUEs:

type: integer

dLThptPerSliceSubnet:

$ref: '#/components/schemas/DLThptPerSliceSubnet'

dLThptPerUEPerSubnet:

$ref: '#/components/schemas/DLThptPerUEPerSubnet'

uLThptPerSliceSubnet:

$ref: '#/components/schemas/ULThptPerSliceSubnet'

uLThptPerUEPerSubnet:

$ref: '#/components/schemas/ULThptPerUEPerSubnet'

maxPktSize:

type: integer

delayTolerance:

$ref: '#/components/schemas/DelayTolerance'

sliceSimultaneousUse:

$ref: '#/components/schemas/SliceSimultaneousUse'

ServiceProfile:

type: object

properties:

serviceProfileId:

type: string

plmnInfoList:

$ref: 'nrNrm.yaml#/components/schemas/PlmnInfoList'

maxNumberofUEs:

type: number

latency:

type: number

uEMobilityLevel:

$ref: '#/components/schemas/MobilityLevel'

sst:

$ref: 'nrNrm.yaml#/components/schemas/Sst'

networkSliceSharingIndicator:

$ref: '#/components/schemas/NetworkSliceSharingIndicator'

availability:

type: number

delayTolerance:

$ref: '#/components/schemas/DelayTolerance'

deterministicComm:

$ref: '#/components/schemas/DeterministicComm'

dLThptPerSlice:

$ref: '#/components/schemas/DLThptPerSlice'

dLThptPerUE:

$ref: '#/components/schemas/DLThptPerUE'

uLThptPerSlice:

$ref: '#/components/schemas/ULThptPerSlice'

uLThptPerUE:

$ref: '#/components/schemas/ULThptPerUE'

maxPktSize:

$ref: '#/components/schemas/MaxPktSize'

maxNumberofPDUSessions:

$ref: '#/components/schemas/MaxNumberofPDUSessions'

kPIMonitoring:

$ref: '#/components/schemas/KPIMonitoring'

nBIoT:

$ref: '#/components/schemas/NBIoT'

synchronicity:

$ref: '#/components/schemas/Synchronicity'

positioning:

$ref: '#/components/schemas/Positioning'

userMgmtOpen:

$ref: '#/components/schemas/UserMgmtOpen'

v2XModels:

$ref: '#/components/schemas/V2XCommModels'

coverageArea:

type: string

termDensity:

$ref: '#/components/schemas/TermDensity'

activityFactor:

$ref: '#/components/schemas/Float'

uESpeed:

type: integer

jitter:

type: integer

survivalTime:

type: string

reliability:

type: string

maxDLDataVolume:

type: string

maxULDataVolume:

type: string

sliceSimultaneousUse:

$ref: '#/components/schemas/SliceSimultaneousUse'

SliceProfile:

type: object

properties:

serviceProfileId:

type: string

plmnInfoList:

$ref: 'nrNrm.yaml#/components/schemas/PlmnInfoList'

perfReq:

$ref: '#/components/schemas/PerfReq'

cNSliceSubnetProfile:

$ref: '#/components/schemas/CNSliceSubnetProfile'

rANSliceSubnetProfile:

$ref: '#/components/schemas/RANSliceSubnetProfile'

topSliceSubnetProfile:

$ref: '#/components/schemas/TopSliceSubnetProfile'

IpAddress:

oneOf:

- $ref: 'genericNrm.yaml#/components/schemas/Ipv4Addr'

- $ref: 'genericNrm.yaml#/components/schemas/Ipv6Addr'

ServiceProfileList:

type: array

items:

$ref: '#/components/schemas/ServiceProfile'

SliceProfileList:

type: array

items:

$ref: '#/components/schemas/SliceProfile'

#------------ Definition of concrete IOCs ----------------------------------------

SubNetwork-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/SubNetwork-Attr'

- $ref: 'genericNrm.yaml#/components/schemas/SubNetwork-ncO'

- type: object

properties:

SubNetwork:

$ref: '#/components/schemas/SubNetwork-Multiple'

NetworkSlice:

$ref: '#/components/schemas/NetworkSlice-Multiple'

NetworkSliceSubnet:

$ref: '#/components/schemas/NetworkSliceSubnet-Multiple'

EP_Transport:

$ref: '#/components/schemas/EP_Transport-Multiple'

NetworkSlice-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/SubNetwork-Attr'

- type: object

properties:

networkSliceSubnetRef:

$ref: 'genericNrm.yaml#/components/schemas/Dn'

operationalState:

$ref: 'genericNrm.yaml#/components/schemas/OperationalState'

administrativeState:

$ref: 'genericNrm.yaml#/components/schemas/AdministrativeState'

serviceProfileList:

$ref: '#/components/schemas/ServiceProfileList'

NetworkSliceSubnet-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/SubNetwork-Attr'

- type: object

properties:

managedFunctionRefList:

$ref: 'genericNrm.yaml#/components/schemas/DnList'

networkSliceSubnetRefList:

$ref: 'genericNrm.yaml#/components/schemas/DnList'

operationalState:

$ref: 'genericNrm.yaml#/components/schemas/OperationalState'

administrativeState:

$ref: 'genericNrm.yaml#/components/schemas/AdministrativeState'

nsInfo:

$ref: '#/components/schemas/NsInfo'

sliceProfileList:

$ref: '#/components/schemas/SliceProfileList'

epTransportRefList:

$ref: 'genericNrm.yaml#/components/schemas/DnList'

EP_Transport-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top-Attr'

- type: object

properties:

attributes:

type: object

properties:

ipAddress:

$ref: '#/components/schemas/IpAddress'

logicInterfaceId:

type: string

nextHopInfo:

type: string

qosProfile:

type: string

epApplicationRefs:

$ref: 'genericNrm.yaml#/components/schemas/DnList'

#-------- Definition of JSON arrays for name-contained IOCs ----------------------

SubNetwork-Multiple:

type: array

items:

$ref: '#/components/schemas/SubNetwork-Single'

NetworkSlice-Multiple:

type: array

items:

$ref: '#/components/schemas/NetworkSlice-Single'

NetworkSliceSubnet-Multiple:

type: array

items:

$ref: '#/components/schemas/NetworkSliceSubnet-Single'

EP_Transport-Multiple:

type: array

items:

$ref: '#/components/schemas/EP_Transport-Single'

#------------ Definitions in TS 28.541 for TS 28.532 -----------------------------

resources-sliceNrm:

oneOf:

- $ref: '#/components/schemas/SubNetwork-Single'

- $ref: '#/components/schemas/NetworkSlice-Single'

- $ref: '#/components/schemas/NetworkSliceSubnet-Single'

- $ref: '#/components/schemas/EP_Transport-Single'
Annex K (normative):
Void

Annex L (normative): 
Relation of GSMA GST, ServiceProfile and SliceProfile

# L.1	General

This annex describes the relation between GSMA GST[50] and information model ServiceProfile and SliceProfile.

# L.2	GSMA GST, ServiceProfile and sliceProfile

The GSMA GST is used as the SLA information for the communication between the NSC (e.g. vertical industry) and the NSP. The SLA requirements can be fulfilled from management aspect and control aspect in a coordinated way. The SLS includes ServiceProfile information model.

As shown in figure L.2.1, the GST [50] is translated and used as input to NRM ServiceProfile, the ServiceProfile can be translated to corresponding requirements for dedicated domains. For example, 5GC SliceProfile is used to carry 5GC domain requirements,  NG-RAN SliceProfile is used to carry NG-RAN domain requirements, and TN requirements are translated and provide to TN domain. Some of the information in 5GC SliceProfile  and NG-RAN SliceProfile translated to configurable parameters  of network function for the control plane SLA support purpose.

Some of the information   (as shown in Table L.2.1) in 5GC SliceProfile and NG-RAN SliceProfile is translated to configurable parameters related to network function behaviour for the control plane SLA support purpose. While other information (e.g delay tolerance, determistic communication support) in 5GC SliceProfile and NG-RAN SliceProfile are kept at OAM domain and is used to determine the overall behaviour of the network slice.

The following table show the translation of GST attributes.

Table L.2.1: GST translation

| GST Attributes | ServiceProfile Parameter | SliceProfile Parameter | Configurable Parameter |
| --- | --- | --- | --- |
| Maximum number of UEs | maxNumberofUEs | maxNumberofUE | TBD |
| Maximum number of PDU sessions | maxNumberofConns | maxNumberofPDUSessions | TBD |
| Downlink maximum throughput per UE | dLThptPerUE | dLThptPerUEPerSubnet | TBD |
| Uplink maximum throughput per UE | uLThptPerUE | uLThptPerUEPerSubnet | TBD |

Editors note: The list of exact configurable parameters is to be revisted depending on the requirements from SA2 and RAN WGs.

NOTE:	Void.

Figure L.2.1 Relation between GSMA GST, ServiceProfile and SliceProfile

Annex M (normative): 
Managed NF Service state handling

# M.1	Combined state diagram for a Managed NF Service

Figure M.1-1: Combined Managed NF Service state diagram

Table M.1-1: The Managed NF Service state transition table

| Trigger number | The state transition events and actions |
| --- | --- |
| 1 | Event: Received information of deployment of a Network Function (NF) service. Action: Create a ManagedNFService instance (MSI) whose(Administrative/Operational/Registration) are set to Locked/Disabled/Deregistered. |
| 2 | Event: Received information of positive state change of the NF service. Action: Set the Operational state of the MSI to Enabled. |
| 3 | Event: Received CM operation to unlock the NF Service or the NF. Action: Set the Administrative state of the MSI to Unlocked. Note: Changing Administrative state on NF service level is optional |
| 4 | Event: Received information that the NF Service is registered to an NRF either by the NF itself or by an OAM system on behalf of the NF. Action: Set the registration state of the MSI to Registered. |
| 5 | Event: Received information that the NF Service is deregistered from the NRF either by the NF itself or by an OAM system on behalf of the NF. Action: Set registration state of the MSI to Deregistered. |
| 6 | Event: Received information that the NF Service is unavailable because of, for example, limitation of resource or other exceptions. Action: Set the Operational state of the MSI to Disabled. |
| 7 | Event: Received information that the NF Service is unavilable. Action: Deregister the NF Service on behalf of the NF, and set the registration state of the MSI to Deregistered. |
| 8 | Event: Received CM operation to lock the NF Service or the NF. Action: Set the Administrative state of the MSI to Locked. Note: Changing Administrative state on NF service level is optional |
| 9 | Event: Received information that the NF Service is terminated or deleted, Action: Delete the MSI and set its state to NULL. |

Annex N (normative):
YANG definition of the Slice NRM

# N.1	General

This annex contains the YANG definitions for the Slice NRM in YANG format.

The Information Service (IS) of the Slice NRM is defined in clause 6.

Mapping rules to produce the YANG definition based on the IS are defined in 3GPP TS 32.160 [14].

# N.2	Modules

## N.2.1	module _3gpp-ns-nrm-networkslice.yang

<CODE BEGINS>

module _3gpp-ns-nrm-networkslice {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-ns-nrm-networkslice;

prefix ns3gpp;

import _3gpp-ns-nrm-networkslicesubnet { prefix nss3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

include _3gpp-ns-nrm-serviceprofile;

organization "3GPP SA5";

contact

"https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "A network slice instance in a 5G network.";

reference "3GPP TS 28.541

Management and orchestration;

5G Network Resource Model (NRM);

Information model definitions for network slice NRM (chapter 6)

";

revision 2020-02-19 {

description "Introduction of YANG definitions for network slice NRM";

reference "CR-0458";

}

grouping NetworkSliceGrp {

uses subnet3gpp:SubNetworkGrp;   // Inherits from SubNetwork

leaf operationalState {

description "The operational state of the network slice instance.

It describes whether or not the resource is physically installed

and working.";

config false;

type types3gpp:OperationalState;

}

leaf administrativeState {

description "The administrative state of the network slice instance.

It describes the permission to use or prohibition against

using the instance, imposed through the OAM services.";

type types3gpp:AdministrativeState;

}

list serviceProfileList {

description "A list of service profiles supported by the network

slice instance.";

key serviceProfileId;

uses ServiceProfileGrp;

}

leaf networkSliceSubnetRef {

type leafref {

path /nss3gpp:NetworkSliceSubnet/nss3gpp:id;

}

description "The NetworkSliceSubnet that the NetworkSlice is

associated with.";

}

}

list NetworkSlice {

description "Represents the properties of a network slice instance in

a 5G network.";

key id;

container attributes {

uses NetworkSliceGrp;

}

uses top3gpp:Top_Grp;

}

}

<CODE ENDS>

## N.2.2	module _3gpp-ns-nrm-networkslicesubnet.yang

<CODE BEGINS>

module _3gpp-ns-nrm-networkslicesubnet {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-ns-nrm-networkslicesubnet;

prefix nss3gpp;

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-measurements { prefix meas3gpp; }

import _3gpp-common-top { prefix top3gpp; }

include _3gpp-ns-nrm-sliceprofile;

organization "3GPP SA5";

contact

"https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the properties of a network slice subnet

instance in a 5G network.";

reference "3GPP TS 28.541

Management and orchestration;

5G Network Resource Model (NRM);

Information model definitions for network slice NRM (chapter 6)

";

revision 2020-02-19 {

description "Introduction of YANG definitions for network slice NRM";

reference "CR-0458";

}

revision 2019-06-07 {

description "initial revision";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

feature MeasurementsUnderNetworkSliceSubnet {

description "The MeasurementSubtree shall be contained under

NetworkSliceSubnet.";

}

typedef ETSI-GS-NFV-Identifier {

type string;

reference "ETSI GS NFV-IFA 013";

}

grouping EPTransportGrp {

leaf ipAddress {

description "This parameter specifies the IP address assigned to a

logical transport interface/endpoint. It can be an IPv4 address

(See RFC 791) or an IPv6 address (See RFC 2373).";

mandatory true;

type string;

}

leaf logicInterfaceId {

description "This parameter specifies the identify of a logical

transport interface. It could be VLAN ID (See IEEE 802.1Q),

MPLS Tag or Segment ID.";

mandatory true;

type string;

}

leaf-list nextHopInfo {

description "This parameter is used to identify ingress transport

node. Each node can be identified by any of combination of IP

address of next-hop router of transport network, system name,

port name, IP management address of transport nodes.";

type string;

}

leaf-list qosProfile {

description "This parameter specifies reference to QoS Profile for

a logical transport interface. A QoS profile includes a set of

parameters which are locally provisioned on both sides of a logical

transport interface.";

type string;

}

leaf-list epApplicationRef {

description "This parameter specifies a list of application level

EPs associated with the logical transport interface.";

min-elements 1;

type types3gpp:DistinguishedName;

}

uses top3gpp:Top_Grp;

}

grouping NsInfoGrp {

description "The NsInfo of the NS instance corresponding to the network

slice subnet instance.";

//suport condition: It shall be supported if the NSS instance is

//realized in the virtualized environment.

// Otherwise this attribute shall be absent.

reference "ETSI GS NFV-IFA 013 clause 8.3.3.2.2, which can be found at

https://www.etsi.org/deliver/etsi_gs/NFV-IFA/001_099/013

/03.04.01_60/gs_NFV-IFA013v030401p.pdf page 123-124";

leaf nSInstanceId {

description "Uniquely identifies the NS instance.";

config false;

type ETSI-GS-NFV-Identifier;

}

leaf nsName {

description "Human readable name of the NS instance.";

type string;

config false;

}

leaf description {

description "Human readable description of the NS instance.";

config false;

type string;

}

}

grouping NetworkSliceSubnetGrp {

uses subnet3gpp:SubNetworkGrp;

uses EPTransportGrp;

leaf operationalState {

description "The operational state of the network slice instance.

It describes whether or not the resource is physically installed

and working.";

mandatory true;

config false;

type types3gpp:OperationalState;

}

leaf administrativeState {

description "The administrative state of the network slice instance.

It describes the permission to use or prohibition against

using the instance, imposed through the OAM services.";

mandatory true;

type types3gpp:AdministrativeState;

}

list nsInfo {

description "This list represents the properties of network service

information corresponding to the network slice subnet instance.";

reference "ETSI GS NFV-IFA 013 clause 8.3.3.2.2";

config false;

key nSInstanceId;

max-elements 1;

uses NsInfoGrp;

}

list sliceProfileList {

description "List of SliceProfiles supported by the network slice

subnet instance";

key sliceProfileId;

uses SliceProfileGrp;

}

list managedFunctionRef {

description "The managed functions that the NetworkSliceSubnet is

associated with.";

key aggregatedManagedFunction;

leaf aggregatedManagedFunction {

type instance-identifier;

}

}

leaf-list networkSliceSubnetRef {

type leafref {

path /NetworkSliceSubnet/id;

}

description "Lists the NetworkSliceSubnet instances associated with

this NetworkSliceSubnet.";

}

}

list NetworkSliceSubnet {

description "Represents the properties of a network slice subnet

instance in a 5G network.";

key id;

container attributes {

uses NetworkSliceSubnetGrp;

leaf-list parents {

description "Reference to direct parent NetworkSliceSubnet

instances.

If NetworkSliceSubnets form a containment hierarchy this is

modeled using references between the child NetworkSliceSubnet

and the parent NetworkSliceSubnet.

This reference MUST NOT be present for the top level

NetworkSliceSubnet and MUST be present for other

NetworkSliceSubnets.";

type leafref {

path "/NetworkSliceSubnet/id";

}

}

leaf-list containedChildren {

description "Reference to all directly contained NetworkSliceSubnet

instances.  If NetworkSliceSubnets form a containment hierarchy

this is modeled using references between the child

NetworkSliceSubnet and the parent NetworkSliceSubnet.";

type leafref {

path "/NetworkSliceSubnet/id";

}

}

}

uses top3gpp:Top_Grp;

uses meas3gpp:MeasurementSubtree {

if-feature MeasurementsUnderNetworkSliceSubnet;

}

}

}

<CODE ENDS>

## N.2.3	module _3gpp-ns-nrm-perfreq.yang

<CODE BEGINS>

module _3gpp-ns-nrm-perfreq {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-ns-nrm-perfreq;

prefix perf3gpp;

organization "3GPP SA5";

contact

"https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "The performance requirements for the NSI in terms of the

scenarios defined in the 3GPP TS 22.261, such as experienced data rate,

area traffic capacity (density) information of UE density.";

reference "3GPP TS 28.541

Management and orchestration;

5G Network Resource Model (NRM);

Information model definitions for network slice NRM (chapter 6)

";

revision 2020-02-19 {

description "Introduction of YANG definitions for network slice NRM";

reference "CR-0458";

}

typedef data-rate {

type uint32;

units kbits/s;

}

typedef integer-percentage {

type uint8 {

range 0..100;

}

units percent;

}

typedef reliability-string {

description "Mean time between failures.

E.g. '1 day', or '3 months'";

type string {

pattern "[0-9]+ (day|week|month|year)s?";

}

reference "3GPP TS 22.104 clause 5.2-5.5";

}

typedef message-size-string {

description "Message size in bytes.

E.g. '80', or '250-2000'";

type string {

pattern '[0-9]+(-[0-9]+)?';

}

units bytes;

reference "3GPP TS 22.104 clause 5.2-5.5";

}

typedef transfer-interval-string {

description "Transfer interval time.  If multiple values are given,

the first value is the application requirement, the other values are

the requirement with multiple transmission of the same information

two or three times, respectively).

E.g. '40ms', or '0ms-5ms,0ms-2.5ms,0ms-1.7ms'";

type string {

pattern '[0-9]+(\.[0-9]+)?m?s-[0-9]+(\.[0-9]+)?m?s' +

'(,[0-9]+(\.[0-9]+)?m?s-[0-9]+(\.[0-9]+)?){0,2}';

}

reference "3GPP TS 22.104 clause 5.2-5.5";

}

typedef survival-time-string {

description "Survival time in milliseconds (ms) or in multiples of

the transfer interval (x).  If multiple values are given,

the first value is the application requirement, the other values are

the requirement with multiple transmission of the same information

two or three times, respectively).

E.g. '12ms', or '0x,2x'";

type string {

pattern '[0-9]+(x|ms)(,[0-9]+(x|ms)){0,2}';

}

reference "3GPP TS 22.104 clause 5.2-5.5";

}

grouping PerfReqGrp {

//Stage2 issue: The perfReq object does not have any proper definition

//              in 28.541 chapter 6.

//Stage2 issue: The text that exists on the perfReq mentions an sST

//              element. There is potentially several sST elements in

//              the SliceProfile/sNSSAIList which could be used as basis

//              for deciding which perfReq elements are relevant.

//              Operators can construct their own sST values. It is not

//              clear which of the perfReq elements below would be

//              relevant in such a case. Leaving all perfReq elements

//              available in all use cases for now.

// eMBB leafs, SST = 1

leaf expDataRateDL {

description "User experienced data rate over downlink";

//TODO: add when 'somepath/sST = 1';

type data-rate;

reference "3GPP TS 22.261 clause 7.1, table 7.1-1";

}

leaf expDataRateUL {

description "User experienced data rate over uplink";

//TODO: add when 'somepath/sST = 1';

type data-rate;

reference "3GPP TS 22.261 clause 7.1, table 7.1-1";

}

leaf areaTrafficCapDL {

description "Area traffic capacity over downlink";

//TODO: add when 'somepath/sST = 1';

type data-rate;

units kbits/s/km2;

reference "3GPP TS 22.261 clause 7.1, table 7.1-1";

}

leaf areaTrafficCapUL {

description "Area traffic capacity over uplink";

//TODO: add when 'somepath/sST = 1';

type data-rate;

units kbits/s/km2;

reference "3GPP TS 22.261 clause 7.1, table 7.1-1";

}

leaf overallUserDensity {

description "Overall user density";

//TODO: add when 'somepath/sST = 1';

type uint32;

units users/km2;

reference "3GPP TS 22.261 clause 7.1, table 7.1-1";

}

leaf activityFactor {

description "Percentage value of the amount of simultaneous active

UEs to the total number of UEs where active means the UEs are

exchanging data with the network.";

//TODO: add when 'somepath/sST = 1';

type integer-percentage;

reference "3GPP TS 22.261 clause 7.1, table 7.1-1";

}

// uRLLC leafs, SST = 2

leaf cSAvailabilityTarget {

description "Reliability uptime target";

//TODO: add when 'somepath/sST = 2';

type decimal64 {

fraction-digits 6; // E.g. 99.999999

range 0..100;

}

reference "3GPP TS 22.104 clause 5.2-5.5";

}

leaf cSReliabilityMeanTime {

description "Mean time between failures";

//TODO: add when 'somepath/sST = 2';

type reliability-string;

}

leaf expDataRate {

description "User experienced data rate";

//TODO: add when 'somepath/sST = 2';

type data-rate;

reference "3GPP TS 22.104 clause 5.2-5.5";

}

leaf msgSizeByte {

description "PDU size";

//TODO: add when 'somepath/sST = 2';

type message-size-string;

}

leaf transferIntervalTarget {

description "Time difference between two consecutive transfers

of application data from an application via the service interface

to 3GPP system";

//TODO: add when 'somepath/sST = 2';

type transfer-interval-string;

}

leaf survivalTime {

description "The time that an application consuming a communication

service may continue without an anticipated message";

//TODO: add when 'somepath/sST = 2';

type survival-time-string;

}

}

}

<CODE ENDS>

## N.2.4	module _3gpp-ns-nrm-serviceprofile.yang

<CODE BEGINS>

submodule _3gpp-ns-nrm-serviceprofile {

yang-version 1.1;

belongs-to _3gpp-ns-nrm-networkslice { prefix ns3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

organization "3GPP SA5";

contact

"https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "A network slice instance in a 5G network.";

reference "3GPP TS 28.541

Management and orchestration;

5G Network Resource Model (NRM);

Information model definitions for network slice NRM (chapter 6)

";

revision 2020-02-19 {

description "Introduction of YANG definitions for network slice NRM";

reference "CR-0458";

}

revision 2019-06-23 {

description "Initial revision";

reference "3GPP TS 28.541 V15.X.XX";

}

typedef availability-percentage {

description "

Percentage value of the amount of time the end-to-end communication

service is delivered according to an agreed QoS, divided by the amount

of time the system is expected to deliver the end-to-end service

according to the specification in a specific area.";

reference "3GPP TS 22.261 3.1";

type decimal64 {

fraction-digits 4; // E.g. 99.9999

range 0..100;

}

}

typedef Category-enum {

type enumeration {

enum character;

enum scalability;

}

}

typedef Tagging-enum {

type enumeration {

enum performance;

enum function;

enum operation;

}

}

typedef Exposure-enum {

type enumeration {

enum API;

enum KPI;

}

}

typedef Support-enum {

type enumeration {

enum NOT_SUPPORTED;

enum SUPPORTED;

}

}

grouping ServAttrComGrp {

leaf category {

description "This attribute specifies the category of a service

requirement/attribute of GST";

type Category-enum;

}

leaf-list tagging {

description "This attribute specifies the tagging of a service

requirement/attribute of GST in character category";

when "../category = 'character'";

type Tagging-enum;

}

leaf exposure {

description "This attribute specifies exposure mode of a service

requirement/attribute of GST";

type Exposure-enum;

}

}

typedef DeterminCommAvailability {

type Support-enum;

}

grouping DLThptGrp {

list servAttrCom {

description "This list represents the common properties of service

requirement related attributes.";

reference "GSMA NG.116 corresponding to Attribute categories,

tagging and exposure";

config false;

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ServAttrComGrp;

}

leaf guaThpt {

description "This attribute describes the guaranteed data rate.";

type uint64;

units kbits/s;

}

leaf maxThpt {

description "This attribute describes the maximum data rate.";

type uint64;

units kbits/s;

}

}

typedef V2XMode-enum {

type enumeration {

enum NOT_SUPPORTED;

enum SUPPORTED_BY_NR;

}

}

grouping ServiceProfileGrp {

leaf serviceProfileId {

description "Service profile identifier.";

type types3gpp:DistinguishedName;

}

list sNSSAIList {

description "The S-NSSAI list to be supported by the new NSI to be

created or the existing NSI to be re-used.";

min-elements 1;

key idx;

unique "sst sd";

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses types5g3gpp:SNssai;

}

list pLMNIdList {

description "List of PLMN IDs.";

min-elements 1;

key "mcc mnc";

ordered-by user;

uses types3gpp:PLMNId;

}

leaf maxNumberofUEs {

description "The maximum number of UEs that may simultaneously

access the network slice instance.";

mandatory true;

type uint64;

}

leaf-list coverageArea {

min-elements 1;

description "A list of TrackingAreas where the NSI can be selected.";

type types3gpp:Tac;

}

leaf latency {

description "The packet transmission latency (milliseconds) through

the RAN, CN, and TN part of 5G network, used to evaluate utilization

performance of the end-to-end network slice instance.";

reference "3GPP TS 28.554 clause 6.3.1";

mandatory true;

type uint16;

units milliseconds;

}

leaf uEMobilityLevel {

description "The mobility level of UE accessing the network slice

instance.";

reference "3GPP TS 22.261 clause 6.2.1";

type types3gpp:UeMobilityLevel;

}

leaf resourceSharingLevel {

description "Specifies whether the resources to be allocated to the

network slice instance may be shared with another network slice

instance(s).";

type types3gpp:ResourceSharingLevel;

}

//Stage2 issue: The sNSSAIList above specifies one or potentially

//              several sST objects for the service profile.

//              How do they relate?

leaf sST {

description "Specifies the slice/service type. See 3GPP TS 23.501

for defined values.";

mandatory true;

type uint32;

reference "3GPP TS 23.501 5.15.2.2";

}

leaf availability {

description "The availability requirement for a network slice

instance, expressed as a percentage.";

type availability-percentage;

}

list delayTolerance {

description "An attribute specifies the properties of service delivery

flexibility, especially for the vertical services that are not

chasing a high system performance.";

reference "TS 22.104 clause 4.3";

config false;

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

list servAttrCom {

description "This list represents the common properties of service

requirement related attributes.";

reference "GSMA NG.116 corresponding to Attribute categories,

tagging and exposure";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ServAttrComGrp;

}

leaf support {

description "An attribute specifies whether or not the network

slice supports service delivery flexibility, especially for the

vertical services that are not chasing a high system performance.";

type Support-enum;

}

}

list deterministicComm {

//Stage2 issue: deterministicComm is not defined in 28.541 chapter 6,

//              but I guess determinComm is meant

description "This list represents the properties of the deterministic

communication for periodic user traffic. Periodic traffic refers to the

type of traffic with periodic transmissions.";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

list servAttrCom {

description "This list represents the common properties of service

requirement related attributes.";

reference "GSMA NG.116 corresponding to Attribute categories,

tagging and exposure";

config false;

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ServAttrComGrp;

}

leaf availability {

//Stage2 issue: Defined differently in 28.541 chapter 6, but XML

//              uses DeterminCommAvailability

config false;

type DeterminCommAvailability;

}

leaf periodicityList {

//Stage2 issue: Not defined in 28.541 chapter 6. XML and YAML

//              says "string".

type string;

}

}

list dLThptPerSlice {

description "This attribute defines achievable data rate of the

network slice in downlink that is available ubiquitously across

the coverage area of the slice";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses DLThptGrp;

}

list dLThptPerUE {

description "This attribute defines data rate supported by the network

slice per UE";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses DLThptGrp;

}

list uLThptPerSlic {

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

description "This attribute defines achievable data rate of the

network slice in uplink that is available ubiquitously across

the coverage area of the slice";

uses DLThptGrp;

}

list uLThptPerUE {

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

description "This attribute defines data rate supported by the

network slice per UE";

uses DLThptGrp;

}

list maxPktSize {

config false;

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

description "This parameter specifies the maximum packet size

supported by the network slice";

list servAttrCom {

description "This list represents the common properties of service

requirement related attributes.";

reference "GSMA NG.116 corresponding to Attribute categories,

tagging and exposure";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ServAttrComGrp;

}

leaf maxSize {

//Stage2 issue: Not defined in 28.541, guessing integer bytes

type uint32;

units bytes;

}

}

list maxNumberofPDUSessions {

description "Represents the maximum number of

concurrent PDU sessions supported by the network slice";

config false;

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

list servAttrCom {

description "This list represents the common properties of service

requirement related attributes.";

reference "GSMA NG.116 corresponding to Attribute categories,

tagging and exposure";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ServAttrComGrp;

}

leaf nOofPDUSessions {

//Stage2 issue: Not defined in 28.541, guessing integer

type uint32;

}

}

list kPIMonitoring {

description "Represents performance monitoring";

config false;

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

list servAttrCom {

description "This list represents the common properties of service

requirement related attributes.";

reference "GSMA NG.116 corresponding to Attribute categories,

tagging and exposure";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ServAttrComGrp;

}

leaf kPIList {

//Stage2 issue: Data format not specified, low interoperability

description "An attribute specifies the name list of KQIs and KPIs

available for performance monitoring";

type string;

}

}

list userMgmtOpen {

description "An attribute specifies whether or not the network slice

supports the capability for the NSC to manage their users or groups

of users’ network services and corresponding requirements.";

config false;

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

list servAttrCom {

description "This list represents the common properties of service

requirement related attributes.";

reference "GSMA NG.116 corresponding to Attribute categories,

tagging and exposure";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ServAttrComGrp;

}

leaf support {

type Support-enum;

}

}

list v2XCommModels {

description "An attribute specifies whether or not the V2X

communication mode is supported by the network slice.";

config false;

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

list servAttrCom {

description "This list represents the common properties of service

requirement related attributes.";

reference "GSMA NG.116 corresponding to Attribute categories,

tagging and exposure";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ServAttrComGrp;

}

leaf v2XMode {

type V2XMode-enum;

}

}

list termDensity {

description "An attribute specifies the overall user density over

the coverage area of the network slice";

config false;

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

list servAttrCom {

description "This list represents the common properties of service

requirement related attributes.";

reference "GSMA NG.116 corresponding to Attribute categories,

tagging and exposure";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ServAttrComGrp;

}

leaf density {

type uint32;

units users/km2;

}

}

leaf activityFactor {

//Stage2 issue: This is modeled as writable/config true in 28.542,

//              but that does not appear to match the description

description "An attribute specifies the percentage value of the

amount of simultaneous active UEs to the total number of UEs where

active means the UEs are exchanging data with the network";

reference "TS 22.261 Table 7.1-1";

type decimal64 {

fraction-digits 1;

}

}

leaf uESpeed {

//Stage2 issue: This is modeled as writable/config true in 28.542,

//              but that does not appear to match the description

description "An attribute specifies the maximum speed (in km/hour)

supported by the network slice at which a defined QoS can be

achieved";

type uint32;

units km/h;

}

leaf jitter {

//Stage2 issue: This is modeled as writable/config true in 28.542,

//              but that does not appear to match the description

description "An attribute specifies the deviation from the desired

value to the actual value when assessing time parameters";

reference "TS 22.104 clause C.4.1";

type uint32;

units microseconds;

}

leaf survivalTime {

description "An attribute specifies the time that an application

consuming a communication service may continue without an

anticipated message.";

reference "TS 22.104 clause 5";

type string;

}

leaf reliability {

description "An attribute specifies in the context of network layer

packet transmissions, percentage value of the amount of sent

network layer packets successfully delivered to a given system

entity within the time constraint required by the targeted service,

divided by the total number of sent network layer packets.";

reference "TS 22.261, TS 22.104";

type string;

}

leaf maxDLDataVolume {

//Stage2 issue: Not defined in 28.541. XML and YAML says "string"

type string;

}

leaf maxULDataVolume {

//Stage2 issue: Not defined in 28.541. XML and YAML says "string"

type string;

}

list nBIoT {

description "An attribute specifies whether NB-IoT is supported in

the RAN in the network slice";

config false;

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

list servAttrCom {

description "This list represents the common properties of service

requirement related attributes.";

reference "GSMA NG.116 corresponding to Attribute categories,

tagging and exposure";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ServAttrComGrp;

}

leaf support {

description "An attribute specifies whether NB-IoT is supported

in the RAN in the network slice";

type Support-enum;

}

}

}

}

<CODE ENDS>

## N.2.5	module _3gpp-ns-nrm-sliceprofile.yang

<CODE BEGINS>

submodule _3gpp-ns-nrm-sliceprofile {

yang-version 1.1;

belongs-to _3gpp-ns-nrm-networkslicesubnet { prefix nss3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-ns-nrm-perfreq { prefix perf3gpp; }

organization "3GPP SA5";

contact

"https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Represents the properties of network slice subnet related

requirement that should be supported by the network slice subnet

instance in a 5G network.";

reference "3GPP TS 28.541

Management and orchestration;

5G Network Resource Model (NRM);

Information model definitions for network slice NRM (chapter 6)

";

revision 2020-02-19 {

description "Introduction of YANG definitions for network slice NRM";

reference "CR-0458";

}

revision 2019-05-27 {

description "initial revision.";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

grouping SliceProfileGrp {

leaf sliceProfileId {

description "A unique identifier of the property of network slice

subnet related requirement should be supported by the network

slice subnet instance.";

type types3gpp:DistinguishedName;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of

supporting. (Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

key idx;

unique "sst sd";

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses types5g3gpp:SNssai;

}

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at

least one (the primary PLMN Id).  The PLMN Identifier is composed

of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

ordered-by user;

uses types3gpp:PLMNId;

}

//Stage2 issue: The perfReq object does not have any proper definition

//              in 28.541 chapter 6.

//Stage2 issue: The text that exists on the perfReq mentions an sST

//              element. There is no sST element in SliceProfile which

//              references perfReq, nor in perfReq itself. There are

//              potentially several in the sNSSAIList. Should we take the

//              union of those to control whivh perfReq elements are

//              relevant? For now, making all perfReq elements available

//              in all slice profiles.

list perfReq {

description "The performance requirements for the NSI in terms of the

scenarios defined in the 3GPP TS 22.261, such as experienced data

rate, area traffic capacity (density) information of UE density.";

key idx; //this list uses a grouping/choice and has no obvious key

leaf idx { type uint32; }

uses perf3gpp:PerfReqGrp;

}

leaf maxNumberofUEs {

description "Specifies the maximum number of UEs may simultaneously

access the network slice instance.";

//optional support

mandatory true;

type uint64;

}

leaf-list coverageAreaTAList {

description "A list of TrackingAreas where the NSI can be selected.";

//optional support

min-elements 1;

type types3gpp:Tac;

}

leaf latency {

description "The packet transmission latency (milliseconds) through

the RAN, CN, and TN part of 5G network, used to evaluate

utilization performance of the end-to-end network slice instance.";

reference "3GPP TS 28.554 clause 6.3.1";

//optional support

mandatory true;

type uint16;

units milliseconds;

}

leaf uEMobilityLevel {

description "The mobility level of UE accessing the network slice

instance.";

//optional support

type types3gpp:UeMobilityLevel;

}

leaf resourceSharingLevel {

description "Specifies whether the resources to be allocated to the

network slice subnet instance may be shared with another network

slice subnet instance(s).";

//optional support

type types3gpp:ResourceSharingLevel;

}

}

}

<CODE ENDS>

Annex O (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2018-09 | SA#81 |  |  |  |  | Upgrade to change control version | 15.0.0 |
| 2018-09 | SA#81 |  |  |  |  | EdiHelp review | 15.0.1 |
| 2018-12 | SA#82 | SP-181046 | 0001 | 1 | F | Fix issues raised by EditHelp | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0002 | 2 | F | Update NR Stage 2 definition to align with TS 37.340 for MR-DC | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0003 | 1 | F | Update NRM Stage 2 defintion to align with TS 23.501 for 5G architecture | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0005 | 1 | F | Update Stage 3 XML definition of NR to align with Stage 2 content | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0006 | 1 | F | Update Stage 3 JSON definition of NR to align with Stage 2 content | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0007 | 1 | F | Update Stage 3 YANG definition of NR to align with Stage 2 content | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0008 | 1 | F | Update Stage 3 XML definition of 5GC to align with Stage 2 content | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0009 | 1 | F | Update Stage 3 JSON definition of 5GC to align with Stage 2 content | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0011 | 1 | F | Update stage 3 XML definition of NS to align with Stage 2 content | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0012 | 1 | F | Update Stage 3 JSON definition of NS to align with Stage 2 content | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0013 | 1 | F | Update stage 3 YANG definition of NS to align with Stage 2 content | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0014 | 1 | F | Correct the term sNSSAIList and nRTAClist | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0015 | 1 | F | Update the inheritance hierarchy figure for NR NRM to include BWP IOC and NRSectorCarrier IOC | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0016 | 1 | F | Change the term nCGI to nCI | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0019 | 1 | F | Align properties of cell state | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0021 | 1 | F | Add missing attribute definition and condition | 15.1.0 |
| 2018-12 | SA#82 | SP-181047 | 0022 | 1 | F | Add missing detail definition for attribute | 15.1.0 |
| 2018-12 | SA#82 | SP-181047 | 0023 | 1 | F | Adding missing attribute, and correction of reference | 15.1.0 |
| 2018-12 | SA#82 | SP-181043 | 0025 | - | F | Remove NSSF from the abbrevations | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0027 | - | F | Replace symbol for network slice state management | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0031 | 1 | F | Remove the ExternalENBFunction definition | 15.1.0 |
| 2018-12 | SA#82 | SP-181046 | 0033 | 1 | F | Align the management of external function and cell with TS 28.658 | 15.1.0 |
| 2018-12 | SA#82 | SP-181156 | 0034 | 1 | F | Update NR NRM with Cell Relation | 15.1.0 |
| 2018-12 | SA#82 | SP-181156 | 0038 | 3 | F | RRM Policy enhancements | 15.1.0 |
| 2018-12 | SA#82 | SP-181156 | 0039 | 1 | F | Fix containment issue in YANG definition | 15.1.0 |
| 2018-12 | SA#82 | SP-181156 | 0040 | - | F | Implement minor corrections | 15.1.0 |
| 2018-12 | SA#82 | SP-181042 | 0041 | - | F | Update Stage 3 NRM for RRM Policy enhancements | 15.1.0 |
| 2019-03 | SA#83 | SP-190121 | 0043 | 1 | F | Align NR attributes definition related to SSB with corresponding NG-RAN IE definition | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0044 | 1 | F | Correct the use of nCI and PLMN | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0045 | - | F | Remove duplicate definition for ExternalNRCellCU | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0046 | 2 | F | Correct class diagram for view on external entities | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0047 | 1 | F | Correct the definition for resourceSharingLevel | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0048 | 1 | F | Correction of references | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0052 | 1 | F | Align the term mFIdList and constituentNSSIIdList | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0053 | 1 | F | Correct the definition of nSSIId | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0054 | 1 | F | Add missing attribute constraint for class definition of NSSFFunction | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0055 | 1 | F | Correct attribute constraints for RRMpolicy related attributes in NRCellCU | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0057 | - | F | Correct cardinality of End Point (EP) to target | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0058 | 0 | F | Correct Import table | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0059 | - | F | Remove ExternalNRCellCU.pLMNIdList | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0060 | - | F | Use 'bS' (not 'bs') to prefix all BS (base station) attributes | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0061 | 1 | F | Correction of State attributes descriptions | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0062 | - | F | Update 5G JSON Solution Set to align with generic NRM | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0063 | 1 | F | Update YANG Solution Set to align with Stage 2 definition | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0064 | 1 | F | Update Information Service to fix Network Slice modeling issue | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0065 | 1 | F | Update Solution Set to fix Network Slice modeling issue | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0066 | 1 | F | Add availability in service profile of network slice resource model | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0068 | 1 | F | Add sST attribute to ServiceProfile | 15.2.0 |
| 2019-03 | SA#83 | SP-190121 | 0069 | 1 | F | Update to sST attribute stage 3 | 15.2.0 |
| 2019-03 | SA#83 | SP-190149 | 0073 | 2 | F | Replace CoverageAreaTAList type definition | 16.0.0 |
| 2019-03 | SA#83 | SP-190149 | 0074 | 1 | F | Name datatypes SliceProfile and ServiceProfile | 16.0.0 |
| 2019-03 | SA#83 | SP-190149 | 0075 | 1 | F | Add datatype definition for S-NSSAI | 16.0.0 |
| 2019-03 | SA#83 | SP-190149 | 0076 | 1 | F | Remove incomplete description for TAC | 16.0.0 |
| 2019-03 | SA#83 | SP-190149 | 0079 | 1 | F | Name datatype RRMPolicyRatio2 | 16.0.0 |
| 2019-06 | SA#84 | SP-190374 | 0083 | - | A | Remove attribute availabilityStatus in NRCellDU IOC | 16.1.0 |
| 2019-06 | SA#84 | SP-190373 | 0085 | 1 | F | Correct the definition for nsInfo | 16.1.0 |
| 2019-06 | SA#84 | SP-190374 | 0088 | 1 | A | Update Information Service of NR to fix unclear Note issue | 16.1.0 |
| 2019-06 | SA#84 | SP-190373 | 0096 | 2 | A | Correct the use of plmnIdList | 16.1.0 |
| 2019-06 | SA#84 | SP-190373 | 0098 | 1 | F | Add missing clauses to RRMPolicyRatio2 data type | 16.1.0 |
| 2019-06 | SA#84 | SP-190373 | 0099 | 1 | F | Update RRMPolicyRatio2 data type name in stage 3 | 16.1.0 |
| 2019-06 | SA#84 | SP-190373 | 0102 | - | F | Fix the implementation errors | 16.1.0 |
| 2019-09 | SA#85 | SP-190745 | 0089 | 2 | B | Update 5GC Information Service to align with Managed Service Definition | 16.2.0 |
| 2019-09 | SA#85 | SP-190743 | 0107 | 1 | A | Correct description for NR deployment scenario | 16.2.0 |
| 2019-09 | SA#85 | SP-190743 | 0109 | 1 | A | Correct NR NRM model to be applicable for all NG-RAN architecture | 16.2.0 |
| 2019-09 | SA#85 | SP-190745 | 0114 | 1 | C | Support NF Profile management | 16.2.0 |
| 2019-09 | SA#85 | SP-190743 | 0121 | 1 | A | Clarification of sNSSAIList attribute | 16.2.0 |
| 2019-09 | SA#85 | SP-190744 | 0123 | - | A | Remove pLMNId from GNBDUFunction | 16.2.0 |
| 2019-09 | SA#85 | SP-190743 | 0126 | 2 | A | Update class definition with inheritance information | 16.2.0 |
| 2019-09 | SA#85 | SP-190743 | 0128 | 1 | A | Correct description of NRCellCU and NRCellDU to be applicable for all deployment scenarios | 16.2.0 |
| 2019-09 | SA#85 | SP-190743 | 0130 | - | A | Correct XML solution set for NR | 16.2.0 |
| 2019-09 | SA#85 | SP-190743 | 0132 | - | A | Correct XML solution set for Network slice | 16.2.0 |
| 2019-09 | SA#85 | SP-190750 | 0133 | 1 | F | Clarification on slice model | 16.2.0 |
| 2019-09 | SA#85 | SP-190743 | 0142 | 1 | A | Add YANG mount info | 16.2.0 |
| 2019-09 | SA#85 | SP-190743 | 0143 | - | A | Add YANG solution | 16.2.0 |
| 2019-09 | SA#85 | SP-190745 | 0149 | 1 | F | generate JSON definition for 5GC NRM based on new style guideline | 16.2.0 |
| 2019-09 | SA#85 | SP-190744 | 0150 | 1 | A | Fix NR NRM to add missed ID info | 16.2.0 |
| 2019-09 | SA#85 | SP-190744 | 0152 | - | F | XML Solution Set for 5GC | 16.2.0 |
| 2019-09 | SA#85 | SP-190744 | 0154 | - | A | Correct ETSI NFV reference | 16.2.0 |
| 2019-09 | SA#85 | SP-190744 | 0157 | 1 | A | generate JSON definition for Slice NRM based on new style guideline | 16.2.0 |
| 2019-09 | SA#85 | SP-190744 | 0158 | 1 | A | generate JSON definition for NR NRM based on new style guideline | 16.2.0 |
| 2019-12 | SA#86 | SP-191159 | 0146 | 3 | F | To syn up with v1540 stage 2 | 16.3.0 |
| 2019-12 | SA#86 | SP-191173 | 0156 | 2 | A | Correct Import table | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0161 | 1 | C | Extensions to PCF and UPF IOCs for support of TSC (Time Sensitive Communication) | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0166 | 1 | F | Correct XML solution set for NR | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0167 | 1 | F | Correct Network slice NRM | 16.3.0 |
| 2019-12 | SA#86 | SP-191173 | 0168 | 2 | A | Correct NR TAC attribute property | 16.3.0 |
| 2019-12 | SA#86 | SP-191173 | 0170 | - | A | Correction of the duplicated IOC NSSFFunction in daigram | 16.3.0 |
| 2019-12 | SA#86 | SP-191173 | 0172 | - | A | Correction of the wrong IOC names in transport view diagram---Not implemented, wrong baseline (MCC) | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0175 | 2 | F | XML Solution Set for 5GC | 16.3.0 |
| 2019-12 | SA#86 | SP-191170 | 0177 | 3 | C | Update on slice NRM | 16.3.0 |
| 2019-12 | SA#86 | SP-191170 | 0178 | 2 | B | Add relation of GST and profiles | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0180 | 3 | F | Update SEPP Stage 2 definition in 5GC NRM | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0182 | 1 | C | Add NEF Stage 2 definition in 5GC NRM | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0184 | 1 | C | Add SCP Stage 2 definition in 5GC NRM | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0185 | - | C | Add Stage 3 definitions of 5GC NRM to align with stage 2 | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0186 | 1 | C | Support communication model in 5GC NF - Stage 2 | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0192 | 1 | F | Fix merging errors of the specification | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0195 | - | C | Add State Handling diagram for NF service | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0197 | - | B | Updates to YANG SS | 16.3.0 |
| 2019-12 | SA#86 | SP-191170 | 0198 | 1 | C | Update XML definitions of ServiceProfile NRM | 16.3.0 |
| 2019-12 | SA#86 | SP-191170 | 0199 | 2 | C | Update JSON definitions of ServiceProfile NRM | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0200 | 1 | C | Add managedNFProfile definition for ngc NRM - stage3 | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0202 | 2 | B | Add the RIM monitoring parameters for remote interference management | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0212 | 2 | F | Correct Network slice NRM | 16.3.0 |
| 2019-12 | SA#86 | SP-191166 | 0213 | - | F | Update SEPP Stage 3 definition in 5GC NRM | 16.3.0 |
| 2019-12 | SA#86 | SP-191180 | 0222 | 2 | B | Management of NR ANR, Stage 2 | 16.3.0 |
| 2019-12 | SA#86 | SP-191180 | 0223 | - | B | Management of NR ANR, Stage 3 | 16.3.0 |
| 2019-12 | SA#86 | SP-191173 | 0226 | 1 | A | Add Stages 2 NRM Info Model definitions for beam managed object classes | 16.3.0 |
| 2019-12 | SA#86 | SP-191173 | 0227 | - | A | Add Stages 2 NRM Info Model definitions for beam managed object classes | 16.3.0 |
| 2020-03 | SA#87E | SP-200169 | 0163 | 4 | F | Correct the parameter sNSSAIList | 16.4.0 |
| 2020-03 | SA#87E | SP-200169 | 0179 | 3 | C | Update of RRM Policy | 16.4.0 |
| 2020-03 | SA#87E | SP-200169 | 0235 | - | F | Correction of reference | 16.4.0 |
| 2020-03 | SA#87E | SP-200169 | 0239 | 1 | F | Update the NR NRM to align with NG-RAN overview architecture | 16.4.0 |
| 2020-03 | SA#87E | SP-200169 | 0241 | - | F | Some correction on the NR NRM | 16.4.0 |
| 2020-03 | SA#87E | SP-200169 | 0242 | - | F | Fix merging errors of the specification | 16.4.0 |
| 2020-03 | SA#87E | SP-200169 | 0243 | 1 | F | Update NRM attribute definitions | 16.4.0 |
| 2020-03 | SA#87E | SP-200233 | 0245 | 2 | B | Add the RIM parameters for remote interference management | 16.4.0 |
| 2020-03 | SA#87E | SP-200234 | 0248 | 1 | F | Update on slice NRM and solution sets | 16.4.0 |
| 2020-03 | SA#87E | SP-200234 | 0250 | 1 | F | Update of GNBCUUPFunction NRM | 16.4.0 |
| 2020-03 | SA#87E | SP-200232 | 0253 | 2 | B | Add Stage 3 NRM Info Model definitions for RRMPolicy and PLMNInfo related CRs | 16.4.0 |
| 2020-03 | SA#87E | SP-200178 | 0254 | 1 | F | Correct CR implementation errors | 16.4.0 |
| 2020-03 | SA#87E | SP-200235 | 0255 | 1 | F | Add OpenAPI definitions required by the ProvMnS | 16.4.0 |
| 2020-03 | SA#87E | SP-200169 | 0258 |  | F | Correct errors in yang solution set | 16.4.0 |
| 2020-03 | SA#87E |  |  |  |  | Correction of implementation errrors | 16.4.1 |
| 2020-06 | SA#88-e | SP-200489 | 0259 | 1 | F | Update on the RRMpolicyRatio | 16.5.0 |
| 2020-06 | SA#88-e | SP-200493 | 0260 | - | F | Replace DN with better identifier for whitelists and blacklists management | 16.5.0 |
| 2020-06 | SA#88-e | SP-200603 | 0261 | 1 | B | Add IOC for control of QoS monitoring per QoS flow per UE | 16.5.0 |
| 2020-06 | SA#88-e | SP-200604 | 0262 | 1 | B | Add IOC for control of GTP-U path QoS monitoring | 16.5.0 |
| 2020-06 | SA#88-e | SP-200489 | 0263 | 1 | F | Correction of reference | 16.5.0 |
| 2020-06 | SA#88-e | SP-200493 | 0268 | - | B | ANR management for EN-DC architecture | 16.5.0 |
| 2020-06 | SA#88-e | SP-200484 | 0269 | 1 | F | Clarification on network slice related identifiers | 16.5.0 |
| 2020-06 | SA#88-e | SP-200484 | 0270 | - | F | Stage 3 update for clarification on network slice related identifiers | 16.5.0 |
| 2020-06 | SA#88-e | SP-200484 | 0274 | 1 | F | Correct sNSSAI definition in XML solution set | 16.5.0 |
| 2020-06 | SA#88-e | SP-200484 | 0275 | 1 | F | Clarify the NR NRM used for different deployment scenarios | 16.5.0 |
| 2020-06 | SA#88-e | SP-200484 | 0278 | - | F | Add missing notification types to the definition of common notifications | 16.5.0 |
| 2020-06 | SA#88-e | SP-200491 | 0279 | 1 | A | Update on NRCellDU | 16.5.0 |
| 2020-06 | SA#88-e | SP-200491 | 0281 | 1 | A | Update Clause 4.2.1.2 Inheritance UML diagram | 16.5.0 |
| 2020-06 | SA#88-e | SP-200490 | 0283 | 2 | B | new NRM fragment to support RIM stage 2 | 16.5.0 |
| 2020-06 | SA#88-e | SP-200490 | 0284 | 1 | B | new NRM fragment to support RIM stage 3 | 16.5.0 |
| 2020-06 | SA#88-e | SP-200489 | 0285 | - | F | Update stage 3 on the RRMpolicyRatio | 16.5.0 |
| 2020-06 | SA#88-e | SP-200605 | 0286 | 2 | B | Add IOC for configurable 5QIs | 16.5.0 |
| 2020-06 | SA#88-e | SP-200490 | 0287 | 1 | B | Add IOC for 5QI to DSCP mapping | 16.5.0 |
| 2020-06 | SA#88-e | SP-200493 | 0289 | - | B | Stage3 add the NRM fragment for SON management | 16.5.0 |
| 2020-06 | SA#88-e | SP-200493 | 0290 | - | B | ANR management for EN-DC architecture | 16.5.0 |
| 2020-06 | SA#88-e | SP-200493 | 0291 | 1 | B | Add the NRM fragment for SON management | 16.5.0 |
| 2020-06 | SA#88-e | SP-200490 | 0293 | - | F | Add CommModelList NRM definition | 16.5.0 |
| 2020-06 | SA#88-e | SP-200490 | 0294 | 1 | F | Update NRM attribute definitions | 16.5.0 |
| 2020-06 | SA#88-e | SP-200490 | 0295 | 1 | F | Correct NRM definition in XML solution | 16.5.0 |
| 2020-06 | SA#88-e | SP-200485 | 0300 | 1 | F | Clarification on the relation of GST, ServiceProfile and SliceProfile | 16.5.0 |
| 2020-06 | SA#88-e | SP-200496 | 0301 | 1 | B | Add ES coverage relation in NRCellRelation | 16.5.0 |
| 2020-06 | SA#88-e | SP-200490 | 0302 | - | F | Update the decription for RRMPolicy_ and resouceType | 16.5.0 |
| 2020-06 | SA#88-e | SP-200490 | 0303 | - | F | Update definition for attribute localAddress in EP_RP IOC | 16.5.0 |
| 2020-06 | SA#88-e | SP-200486 | 0305 | 1 | A | Correction of references | 16.5.0 |
| 2020-06 | SA#88-e | SP-200485 | 0306 | 1 | F | add transport information and slice mapping on backhaul endpoints | 16.5.0 |
| 2020-06 | SA#88-e | SP-200485 | 0307 | - | F | add transport information and slice mapping on backhaul endpoints stage 3 | 16.5.0 |
| 2020-06 | SA#88-e | SP-200490 | 0312 | 1 | F | Update SliceProfile attributes solution 1 | 16.5.0 |
| 2020-06 | SA#88-e | SP-200490 | 0315 | 1 | B | Add configuredMaxTxEIRP on NRSectorCarrier | 16.5.0 |
| 2020-06 | SA#88-e | SP-200490 | 0316 | - | B | Stage 3 Add configuredMaxTxEIRP on NRSectorCarrier | 16.5.0 |
| 2020-06 | SA#88-e | SP-200490 | 0318 | - | F | Update NRM YANG for 28.541 | 16.5.0 |
| 2020-06 | SA#88-e | SP-200496 | 0319 | - | B | Add ES coverage relation in NRCellRelation Stage 3 | 16.5.0 |
| 2020-06 | SA#88-e | SP-200612 | 0320 | 1 | F | Update openAPI for NRCellRelation and NRFreqRelation | 16.5.0 |
| 2020-09 | SA#89-e | SP-200729 | 0321 | - | F | Correction of NRM YANG errors | 16.6.0 |
| 2020-09 | SA#89-e | SP-200729 | 0322 | 1 | F | Correct on NR NRM | 16.6.0 |
| 2020-09 | SA#89-e | SP-200729 | 0323 | - | F | Correct the openAPI definition for NR NRM | 16.6.0 |
| 2020-09 | SA#89-e | SP-200730 | 0325 | - | A | Correct on frequency related IOC | 16.6.0 |
| 2020-09 | SA#89-e | SP-200729 | 0329 | 1 | B | Add IOC for predefined PCC rules | 16.6.0 |
| 2020-09 | SA#89-e | SP-200729 | 0330 | 2 | B | Add IOC for predefined PCC rules | 16.6.0 |
| 2020-09 | SA#89-e | SP-200729 | 0331 | - | B | Enable PCF to support configurable 5QIs | 16.6.0 |
| 2020-09 | SA#89-e | SP-200729 | 0332 | - | B | Add IOC for dynamic 5QIs - stage 2 | 16.6.0 |
| 2020-09 | SA#89-e | SP-200729 | 0333 | - | B | Add IOC for dynamic 5QIs - stage 3 | 16.6.0 |
| 2020-09 | SA#89-e | SP-200729 | 0334 | - | B | Add TCE mapping info in GNBCUCPFunction | 16.6.0 |
| 2020-09 | SA#89-e | SP-200729 | 0335 | - | B | Add TCE mapping info in openAPI solution | 16.6.0 |
| 2020-09 | SA#89-e | SP-200729 | 0336 | - | F | Add missing definitions for perfReq | 16.6.0 |
| 2020-09 | SA#89-e | SP-200754 | 0338 | 1 | F | Delete supportedAccessTech to align with GST | 16.6.0 |
| 2020-09 | SA#89-e | SP-200724 | 0339 | - | F | Correction on duplicated annex numbering | 16.6.0 |
| 2020-09 | SA#89-e | SP-200729 | 0345 | - | F | Update NRM attribute definitions | 16.6.0 |
| 2020-09 | SA#89-e | SP-200749 | 0362 | - | F | Deleting SupportedAccessTech - Stage 3 - XML | 16.6.0 |
| 2020-09 | SA#89-e | SP-200724 | 0368 | 1 | F | Add relation between transport and application level endpoints | 16.6.0 |
| 2020-09 | SA#89-e | SP-200724 | 0369 | - | F | Add relation between transport and application level endpoints stage 3 | 16.6.0 |
| 2020-09 | SA#89-e | SP-200729 | 0370 | 1 | F | Cleanup stage 2 editorial issue and stage 3 yaml error | 16.6.0 |
| 2020-09 | SA#89-e | SP-200749 | 0371 | - | F | Add clarifying note to ServiceProfile | 16.6.0 |
| 2020-09 | SA#89-e | SP-200752 | 0337 | - | B | Add the MLB support indicator in NRcellrelation | 17.0.0 |
| 2020-09 | SA#89-e | SP-200749 | 0341 | 1 | F | Update maxNumberofConns | 17.0.0 |
| 2020-09 | SA#89-e | SP-200749 | 0342 | - | B | Add NB-IoT support in ServiceProfile | 17.0.0 |
| 2020-09 | SA#89-e | SP-200729 | 0366 | 1 | B | Addition of attribute for network slice supporting maximum of data volume | 17.0.0 |
| 2020-11 |  |  |  |  |  | No technical changes. Cleanup of diverse issues in order to improve performance of the file: hidden XML, watermarks,etc.. | 17.0.1 |
| 2020-12 | SA#90e | SP-201057 | 0380 | - | A | Correct the definition for configurable5QI and dynamic5QI | 17.1.0 |
| 2020-12 | SA#90e | SP-201066 | 0382 | 1 | F | Change RACH control attributes from beam to cell | 17.1.0 |
| 2020-12 | SA#90e | SP-201045 | 0384 | 1 | A | Move Distributed RACH control IOC from CU to DU | 17.1.0 |
| 2020-12 | SA#90e | SP-201045 | 0386 | 2 | A | Move Distributed PCI control IOC from DU to CU | 17.1.0 |
| 2020-12 | SA#90e | SP-201057 | 0388 | - | A | Correction of cell neighbour relations related attributes in openAPI solution | 17.1.0 |
| 2020-12 | SA#90e | SP-201057 | 0393 | - | A | Correction of NRM YANG errors | 17.1.0 |
| 2020-12 | SA#90e | SP-201057 | 0395 | 1 | A | Correct Network slice NRM | 17.1.0 |
| 2020-12 | SA#90e | SP-201053 | 0399 | 1 | A | Fix description related to service profile | 17.1.0 |
| 2020-12 | SA#90e | SP-201050 | 0405 | - | A | Add containment relationship for network slice IOCs | 17.1.0 |
| 2020-12 | SA#90e | SP-201050 | 0406 | - | F | Add containment relationship for network slice IOCs stage 3 | 17.1.0 |
| 2020-12 | SA#90e | SP-201045 | 0407 | - | F | Add subclause reference of MRO related attribute | 17.1.0 |
| 2020-12 | SA#90e | SP-201089 | 0410 | 1 | A | Correction of NRM YANG errors | 17.1.0 |
| 2020-12 | SA#90e | SP-201089 | 0412 | - | A | YANG improvements | 17.1.0 |
| 2020-12 | SA#90e | SP-201056 | 0414 | - | A | Add serviceProfileId and sliceProfileId to stage 3 yaml | 17.1.0 |
| 2020-12 | SA#90e | SP-201089 | 0419 | - | A | Update notifyThresholdCrossing to be a common notification. | 17.1.0 |
| 2020-12 | SA#90e | SP-201089 | 0421 | - | A | pLMNInfoList faulty attribute definition | 17.1.0 |
| 2020-12 | SA#90e | SP-201089 | 0423 | - | A | Fix containment relationship for EP_Transport IOC | 17.1.0 |
| 2021-03 | SA#91e | SP-210153 | 0432 | 1 | A | Correction on Dynamic5QISet IOC based on LS reply from SA2 | 17.2.0 |
| 2021-03 | SA#91e | SP-210154 | 0435 | 3 | A | Correct the NF name in definition of EP_NgU | 17.2.0 |
| 2021-03 | SA#91e | SP-210153 | 0440 | - | A | Add missing inheritance description information in the attribute definition for several IOCs | 17.2.0 |
| 2021-03 | SA#91e | SP-210153 | 0442 | 2 | A | Correct multiplicity issue for several attributes of NR NRM | 17.2.0 |
| 2021-03 | SA#91e | SP-210146 | 0445 | 2 | A | Fix containment relationship for EP_Transport IOC | 17.2.0 |
| 2021-03 | SA#91e | SP-210155 | 0457 | - | C | Remove the XML Solution set | 17.2.0 |
| 2021-03 | SA#91e | SP-210144 | 0459 | 1 | B | Update the information model definitions for network slice NRM | 17.2.0 |
| 2021-03 | SA#91e | SP-210143 | 0461 | 1 | A | Update of the PCI and DESManagementFunction | 17.2.0 |
| 2021-03 | SA#91e | SP-210154 | 0467 | 1 | A | Correction to NSI and NSSI state management | 17.2.0 |
| 2021-03 | SA#91e | SP-210155 | 0472 | - | A | YANG compilation error and missing stage 2 corrections | 17.2.0 |
| 2021-03 | SA#91e | SP-210146 | 0474 | - | A | Fix compilation and other errors | 17.2.0 |
| 2021-03 | SA#91e |  |  |  |  | Fixing CR implementation error in E.5.13 | 17.2.1 |