| 3GPP TS 28.541 V17.4.0 (2021-09) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3 (Release 17 ) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	24

Introduction	25

1	Scope	26

2	References	26

3	Definitions of terms, symbols and abbreviations	29

3.1	Terms	29

3.2	Symbols	29

3.3	Abbreviations	29

4	Information model definitions for NR NRM	29

4.1	Imported and associated information	29

4.1.1	Imported information entities and local labels	29

4.1.2	Associated information entities and local labels	29

4.2	Class diagram	29

4.2.1	Class diagram for gNB and en-gNB	29

4.2.1.1	Relationships	29

4.2.1.2	Inheritance	35

4.3	Class definitions	37

4.3.1	GNBDUFunction	37

4.3.1.1	Definition	37

4.3.1.2	Attributes	37

4.3.1.3	Attribute constraints	37

4.3.1.4	Notifications	38

4.3.2	GNBCUCPFunction	38

4.3.2.1	Definition	38

4.3.2.2	Attributes	38

4.3.2.3	Attribute constraints	39

4.3.2.4	Notifications	39

4.3.3	GNBCUUPFunction	39

4.3.3.1	Definition	39

4.3.3.2	Attributes	39

4.3.3.3	Attribute constraints	40

4.3.3.4	Notifications	40

4.3.4	NRCellCU	40

4.3.4.1	Definition	40

4.3.4.2	Attributes	40

4.3.4.3	Void	40

4.3.4.4	Notifications	40

4.3.5	NRCellDU	40

4.3.5.1	Definition	40

4.3.5.2	Attributes	41

4.3.5.3	Attribute constraints	42

4.3.5.4	Notifications	42

4.3.6	NRSectorCarrier	43

4.3.6.1	Definition	43

4.3.6.2	Attributes	43

4.3.6.3	Attribute constraints	43

4.3.6.4	Notifications	43

4.3.7	BWP	44

4.3.7.1	Definition	44

4.3.7.2	Attributes	44

4.3.7.3	Attribute constraints	44

4.3.7.4	Notifications	44

4.3.8	EP_E1	44

4.3.8.1	Definition	44

4.3.8.2	Attributes	44

4.3.8.3	Attribute constraints	44

4.3.8.4	Notifications	45

4.3.9	EP_XnU	45

4.3.9.1	Definition	45

4.3.9.2	Attributes	45

4.3.9.3	Attribute constraints	45

4.3.9.4	Notifications	45

4.3.10	EP_NgC	45

4.3.10.1	Definition	45

4.3.10.2	Attributes	45

4.3.10.3	Attribute constraints	46

4.3.10.4	Notifications	46

4.3.11	EP_NgU	46

4.3.11.1	Definition	46

4.3.11.2	Attributes	46

4.3.11.3	Attribute constraints	46

4.3.11.4	Notifications	46

4.3.12	EP_F1C	46

4.3.12.1	Definition	46

4.3.12.2	Attributes	46

4.3.12.3	Attribute constraints	47

4.3.12.4	Notifications	47

4.3.13	EP_F1U	47

4.3.13.1	Definition	47

4.3.13.2	Attributes	47

4.3.13.3	Attribute constraints	47

4.3.13.4	Notifications	47

4.3.14	EP_S1U	47

4.3.14.1	Definition	47

4.3.14.2	Attributes	48

4.3.14.3	Attribute constraints	48

4.3.14.4	Notifications	48

4.3.15	EP_X2C	48

4.3.15.1	Definition	48

4.3.15.2	Attributes	48

4.3.15.3	Attribute constraints	48

4.3.15.4	Notifications	48

4.3.16	EP_X2U	48

4.3.16.1	Definition	48

4.3.16.2	Attributes	49

4.3.16.3	Attribute constraints	49

4.3.16.4	Notifications	49

4.3.17	EP_XnC	49

4.3.17.1	Definition	49

4.3.17.2	Attributes	49

4.3.17.3	Attribute constraints	49

4.3.17.4	Notifications	49

4.3.18	ExternalGNBCUCPFunction	49

4.3.18.1	Definition	49

4.3.18.2	Attributes	50

4.3.18.3	Attribute constraints	50

4.3.18.4	Notifications	50

4.3.19	ExternalGNBCUUPFunction	50

4.3.19.1	Definition	50

4.3.19.2	Attributes	50

4.3.19.3	Attribute constraints	50

4.3.19.4	Notifications	50

4.3.20	ExternalGNBDUFunction	50

4.3.20.1	Definition	50

4.3.20.2	Attributes	51

4.3.20.3	Attribute constraints	51

4.3.20.4	Notifications	51

4.3.21	ExternalUPFFunction	51

4.3.21.1	Definition	51

4.3.21.2	Attributes	51

4.3.21.3	Attribute constraints	51

4.3.21.4	Notifications	51

4.3.22	ExternalAMFFunction	51

4.3.22.1	Definition	51

4.3.22.2	Attributes	52

4.3.22.3	Attribute constraints	52

4.3.22.4	Notifications	52

4.3.23	Void	52

4.3.24	ENBFunction <<ProxyClass>>	52

4.3.24.1	Definition	52

4.3.24.2	Attributes	52

4.3.24.3	Attribute constraints	52

4.3.24.4	Notifications	52

4.3.25	GNBCUCPFunction <<ProxyClass>>	52

4.3.25.1	Definition	52

4.3.25.2	Attributes	52

4.3.25.3	Attribute constraints	52

4.3.25.4	Notifications	53

4.3.26	GNBCUUPFunction <<ProxyClass>>	53

4.3.26.1	Definition	53

4.3.26.2	Attributes	53

4.3.26.3	Attribute constraints	53

4.3.26.4	Notifications	53

4.3.27	GNBDUFunction <<ProxyClass>>	53

4.3.27.1	Definition	53

4.3.27.2	Attributes	53

4.3.27.3	Attribute constraints	53

4.3.27.4	Notifications	53

4.3.28	ServingGWFFunction <<ProxyClass>>	53

4.3.28.1	Definition	53

4.3.28.2	Attributes	53

4.3.28.3	Attribute constraints	53

4.3.28.4	Notifications	54

4.3.29	UPFFunction <<ProxyClass>>	54

4.3.29.1	Definition	54

4.3.29.2	Attributes	54

4.3.29.3	Attribute constraints	54

4.3.29.4	Notifications	54

4.3.30	AMFFunction <<ProxyClass>>	54

4.3.30.1	Definition	54

4.3.30.2	Attributes	54

4.3.30.3	Attribute constraints	54

4.3.30.4	Notifications	54

4.3.31	Void	54

4.3.32	NRCellRelation	54

4.3.32.1	Definition	54

4.3.32.2	Attributes	55

4.3.32.3	Attribute constraints	55

4.3.32.4	Notifications	55

4.3.33	NRFreqRelation	55

4.3.33.1	Definition	55

4.3.33.2	Attributes	55

4.3.33.3	Attribute constraints	56

4.3.33.4	Notifications	56

4.3.34	Void	56

4.3.35	ExternalNRCellCU	56

4.3.35.1	Definition	56

4.3.35.2	Attributes	56

4.3.35.3	Attribute constraints	57

4.3.35.4	Notifications	57

4.3.36	RRMPolicyRatio	57

4.3.36.1	Definition	57

4.3.36.2	Attributes	58

4.3.36.3	Attribute constraints	58

4.3.36.4	Notifications	58

4.3.37	S-NSSAI <<dataType>>	58

4.3.37.1	Definition	58

4.3.37.2	Attributes	58

4.3.37.3	Attribute constraints	59

4.3.37.4	Notifications	59

4.3.38	NRFrequency	59

4.3.38.1	Definition	59

4.3.38.2	Attributes	59

4.3.38.3	Attribute constraints	59

4.3.38.4	Notifications	59

4.3.39	CommonBeamformingFunction	59

4.3.39.1	Definition	59

4.3.39.2	Attributes	59

4.3.39.3	Attribute constraints	60

4.3.39.4	Notifications	60

4.3.40	Beam	60

4.3.40.1	Definition	60

4.3.40.2	Attributes	60

4.3.40.3	Attribute constraints	60

4.3.41	PLMNInfo <<dataType>>	61

4.3.41.1	Definition	61

4.3.41.2	Attributes	61

4.3.41.3	Attribute constraints	61

4.3.41.4	Notifications	61

4.3.42	RRMPolicyMember <<dataType>>	61

4.3.42.1	Definition	61

4.3.42.2	Attributes	61

4.3.42.3	Attribute constraints	62

4.3.42.4	Notifications	62

4.3.43	RRMPolicy_	62

4.3.43.1	Definition	62

4.3.43.2	Attributes	62

4.3.43.3	Attribute constraints	62

4.3.43.4	Notifications	62

4.3.44	RRMPolicyManagedEntity <<ProxyClass>>	62

4.3.44.1	Definition	62

4.3.44.2	Attributes	63

4.3.44.3	Attribute constraints	63

4.3.44.4	Notifications	63

4.3.45	GNBCUCPNeighbour <<ProxyClass>>	63

4.3.45.1	Definition	63

4.3.45.2	Attributes	63

4.3.45.3	Attribute constraints	63

4.3.45.4	Notifications	63

4.3.46	GNBCUUPNeighbour <<ProxyClass>>	63

4.3.46.1	Definition	63

4.3.46.2	Attributes	64

4.3.46.3	Attribute constraints	64

4.3.46.4	Notifications	64

4.3.47	MappingSetIDBackhaulAddress  <<dataType>>	64

4.3.47.1	Definition	64

4.3.47.2	Attributes	64

4.3.47.3	Attribute constraints	64

4.3.47.4	Notifications	64

4.3.48	BackhaulAddress  <<dataType>>	64

4.3.48.1	Definition	64

4.3.48.2	Attributes	64

4.3.48.3	Attribute constraints	65

4.3.48.4	Notifications	65

4.3.49	TAI <<dataType>>	65

4.3.49.1	Definition	65

4.3.49.2	Attributes	65

4.3.49.3	Attribute constraints	65

4.3.49.4	Notifications	65

4.3.50	RimRSGlobal	65

4.3.50.1	Definition	65

4.3.50.2	Attributes	65

4.3.50.3	Attribute constraints	65

4.3.50.4	Notifications	66

4.3.51	FrequencyDomainPara <<dataType>>	66

4.3.51.1	Definition	66

4.3.51.2	Attributes	66

4.3.51.3	Attribute constraints	66

4.3.51.4	Notifications	66

4.3.52	SequenceDomainPara  <<dataType>>	66

4.3.52.1	Definition	66

4.3.52.2	Attributes	66

4.3.52.3	Attribute constraints	67

4.3.52.4	Notifications	67

4.3.53	TimeDomainPara  <<dataType>>	67

4.3.53.1	Definition	67

4.3.53.2	Attributes	67

4.3.53.3	Attribute constraints	68

4.3.53.4	Notifications	68

4.3.54	RimRSReportConf <<dataType>>	68

4.3.54.1	Definition	68

4.3.54.2	Attributes	68

4.3.54.3	Attribute constraints	68

4.3.54.4	Notifications	68

4.3.55	RimRSReportInfo <<dataType>>	68

4.3.55.1	Definition	68

4.3.55.2	Attributes	69

4.3.55.3	Attribute constraints	69

4.3.55.4	Notifications	69

4.3.56	RimRSSet	69

4.3.56.1	Definition	69

4.3.56.2	Attributes	69

4.3.56.3	Attribute constraints	69

4.3.56.4	Notifications	69

4.3.57	DANRManagementFunction	69

4.3.57.1	Definition	69

4.3.57.2	Attributes	70

4.3.57.3	Attribute constraints	70

4.3.57.4	Notifications	70

4.3.58	DESManagementFunction	70

4.3.58.1	Definition	70

4.3.58.2	Attributes	70

4.3.58.3	Attribute constraints	71

4.3.58.4	Notification	71

4.3.59	DRACHOptimizationFunction	71

4.3.59.1	Definition	71

4.3.59.2	Attributes	71

4.3.59.3	Attribute constraints	71

4.3.59.4	Notifications	71

4.3.60	DMROFunction	72

4.3.60.1	Definition	72

4.3.60.2	Attributes	72

4.3.60.3	Attribute constraints	72

4.3.60.4	Notifications	72

4.3.61	DPCIConfigurationFunction	72

4.3.61.1	Definition	72

4.3.61.2	Attributes	72

4.3.61.3	Attribute constraints	72

4.3.61.4	Notifications	73

4.3.62	CPCIConfigurationFunction	73

4.3.62.1	Definition	73

4.3.62.2	Attributes	73

4.3.62.3	Attribute constraints	73

4.3.62.4	Notifications	73

4.3.63	CESManagementFunction	73

4.3.63.1	Definition	73

4.3.63.2	Attributes	73

4.3.63.3	Attribute constraints	74

4.3.63.4	Notification	74

4.3.64	AddressWithVlan <<dataType>>	74

4.3.64.1	Definition	74

4.3.64.2	Attributes	75

4.3.64.3	Attribute constraints	75

4.3.64.4	Notifications	75

4.3.65	TceIDMappingInfo  <<dataType>>	75

4.3.65.1	Definition	75

4.3.65.2	Attributes	75

4.3.65.3	Attribute constraints	75

4.3.65.4	Notifications	75

4.3.66	NPNIdentity <<dataType>>	75

4.3.66.1	Definition	75

4.3.66.2	Attributes	76

4.3.66.3	Attribute constraints	76

4.3.66.4	Notifications	76

4.4	Attribute definitions	77

4.4.1	Attribute properties	77

4.5	Common notifications	106

4.5.1	Alarm notifications	106

4.5.2	Configuration notifications	106

4.5.3	Threshold Crossing notifications	107

5	Information Model definitions for 5GC NRM	107

5.1	Imported information entities and local labels	107

5.2	Class diagram	107

5.2.1	Class diagram of 5GC NFs	107

5.2.1.1	Relationships	107

5.2.1.2	Inheritance	115

5.2.2	Class diagram of AMF Region/AMF Set	118

5.2.2.1	Relationships	118

5.2.2.2	Inheritance	119

5.3	Class definitions	119

5.3.1	AMFFunction	119

5.3.1.1	Definition	119

5.3.1.2	Attributes	119

5.3.1.3	Attribute constraints	120

5.3.1.4	Notifications	120

5.3.2	SMFFunction	120

5.3.2.1	Definition	120

5.3.2.2	Attributes	120

5.3.2.3	Attribute constraints	121

5.3.2.4	Notifications	121

5.3.3	UPFFunction	121

5.3.3.1	Definition	121

5.3.3.2	Attributes	121

5.3.3.3	Attribute constraints	121

5.3.3.4	Notifications	121

5.3.4	N3IWFFunction	121

5.3.4.1	Definition	121

5.3.4.2	Attributes	121

5.3.4.3	Attribute constraints	122

5.3.4.4	Notifications	122

5.3.5	PCFFunction	122

5.3.5.1	Definition	122

5.3.5.2	Attributes	122

5.3.5.3	Attribute constraints	122

5.3.5.4	Notifications	122

5.3.6	AUSFFunction	123

5.3.6.1	Definition	123

5.3.6.2	Attributes	123

5.3.6.3	Attribute constraints	123

5.3.6.4	Notifications	123

5.3.7	UDMFunction	123

5.3.7.1	Definition	123

5.3.7.2	Attributes	123

5.3.5.3	Attribute constraints	124

5.3.5.4	Notifications	124

5.3.8	UDRFunction	124

5.3.8.1	Definition	124

5.3.8.2	Attributes	124

5.3.8.3	Attribute constraints	124

5.3.8.4	Notifications	124

5.3.9	UDSFFunction	124

5.3.9.1	Definition	124

5.3.9.2	Attributes	124

5.3.9.3	Attribute constraints	125

5.3.9.4	Notifications	125

5.3.10	NRFFunction	125

5.3.10.1	Definition	125

5.3.10.2	Attributes	125

5.3.10.3	Attribute constraints	125

5.3.10.4	Notifications	125

5.3.11	NSSFFunction	126

5.3.11.1	Definition	126

5.3.11.2	Attributes	126

5.3.11.3	Attribute constraints	126

5.3.11.4	Notifications	126

5.3.12	AFFunction	126

5.3.12.1	Definition	126

5.3.13	DNFunction	126

5.3.13.1	Definition	126

5.3.14	SMSFFunction	126

5.3.14.1	Definition	126

5.3.14.2	Attributes	126

5.3.14.3	Attribute constraints	127

5.3.14.4	Notifications	127

5.3.15	LMFFunction	127

5.3.15.1	Definition	127

5.3.15.2	Attributes	127

5.3.15.3	Attribute constraints	127

5.3.15.4	Notifications	127

5.3.16	NGEIRFunction	127

5.3.16.1	Definition	127

5.3.16.2	Attributes	127

5.3.16.3	Attribute constraints	128

5.3.16.4	Notifications	128

5.3.17	SEPPFunction	128

5.3.17.1	Definition	128

5.3.17.2	Attributes	128

5.3.17.3	Attribute constraints	128

5.3.17.4	Notifications	128

5.3.18	NWDAFFunction	128

5.3.18.1	Definition	128

5.3.18.2	Attributes	129

5.3.18.3	Attribute constraints	129

5.3.18.4	Notifications	129

5.3.19	EP_N2	129

5.3.19.1	Definition	129

5.3.19.2	Attributes	129

5.3.19.3	Attribute constraints	129

5.3.19.4	Notifications	129

5.3.20	EP_N3	130

5.3.20.1	Definition	130

5.3.20.2	Attributes	130

5.3.20.3	Attribute constraints	130

5.3.20.4	Notifications	130

5.3.21	EP_N4	130

5.3.21.1	Definition	130

5.3.21.2	Attributes	130

5.3.21.3	Attribute constraints	130

5.3.21.4	Notifications	130

5.3.22	EP_N5	131

5.3.22.1	Definition	131

5.3.22.2	Attributes	131

5.3.22.3	Attribute constraints	131

5.3.22.4	Notifications	131

5.3.23	EP_N6	131

5.3.23.1	Definition	131

5.3.23.2	Attributes	131

5.3.23.3	Attribute constraints	131

5.3.23.4	Notifications	131

5.3.24	EP_N7	131

5.3.24.1	Definition	131

5.3.24.2	Attributes	132

5.3.24.3	Attribute constraints	132

5.3.24.4	Notifications	132

5.3.25	EP_N8	132

5.3.25.1	Definition	132

5.3.25.2	Attributes	132

5.3.25.3	Attribute constraints	132

5.3.25.4	Notifications	132

5.3.26	EP_N9	132

5.3.26.1	Definition	132

5.3.26.2	Attributes	132

5.3.26.3	Attribute constraints	133

5.3.26.4	Notifications	133

5.3.27	EP_N10	133

5.3.27.1	Definition	133

5.3.27.2	Attributes	133

5.3.27.3	Attribute constraints	133

5.3.27.4	Notifications	133

5.3.28	EP_N11	133

5.3.28.1	Definition	133

5.3.28.2	Attributes	133

5.3.28.3	Attribute constraints	134

5.3.28.4	Notifications	134

5.3.29	EP_N12	134

5.3.29.1	Definition	134

5.3.29.2	Attributes	134

5.3.29.3	Attribute constraints	134

5.3.29.4	Notifications	134

5.3.30	EP_N13	134

5.3.30.1	Definition	134

5.3.30.2	Attributes	134

5.3.30.3	Attribute constraints	135

5.3.30.4	Notifications	135

5.3.31	EP_N14	135

5.3.31.1	Definition	135

5.3.31.2	Attributes	135

5.3.31.3	Attribute constraints	135

5.3.31.4	Notifications	135

5.3.32	EP_N15	135

5.3.32.1	Definition	135

5.3.32.2	Attributes	135

5.3.32.3	Attribute constraints	136

5.3.32.4	Notifications	136

5.3.33	EP_N16	136

5.3.33.1	Definition	136

5.3.33.2	Attributes	136

5.3.33.3	Attribute constraints	136

5.3.33.4	Notifications	136

5.3.34	EP_N17	136

5.3.34.1	Definition	136

5.3.34.2	Attributes	136

5.3.34.3	Attribute constraints	137

5.3.34.4	Notifications	137

5.3.35	EP_N20	137

5.3.35.1	Definition	137

5.3.35.2	Attributes	137

5.3.35.3	Attribute constraints	137

5.3.35.4	Notifications	137

5.3.36	EP_N21	137

5.3.36.1	Definition	137

5.3.36.2	Attributes	137

5.3.36.3	Attribute constraints	138

5.3.36.4	Notifications	138

5.3.37	EP_N22	138

5.3.37.1	Definition	138

5.3.37.2	Attributes	138

5.3.37.3	Attribute constraints	138

5.3.37.4	Notifications	138

5.3.38	EP_N26	138

5.3.38.1	Definition	138

5.3.38.2	Attributes	138

5.3.38.3	Attribute constraints	139

5.3.38.4	Notifications	139

5.3.39	Void	139

5.3.40	Void	139

5.3.41	EP_S5C	139

5.3.41.1	Definition	139

5.3.41.2	Attributes	139

5.3.41.3	Attribute constraints	139

5.3.41.4	Notifications	139

5.3.42	EP_S5U	139

5.3.42.1	Definition	139

5.3.42.2	Attributes	139

5.3.42.3	Attribute constraints	140

5.3.42.4	Notifications	140

5.3.43	EP_Rx	140

5.3.43.1	Definition	140

5.3.43.2	Attributes	140

5.3.43.3	Attribute constraints	140

5.3.43.4	Notifications	140

5.3.44	EP_MAP_SMSC	140

5.3.44.1	Definition	140

5.3.44.2	Attributes	140

5.3.44.3	Attribute constraints	141

5.3.44.4	Notifications	141

5.3.45	EP_NLS	141

5.3.45.1	Definition	141

5.3.45.2	Attributes	141

5.3.45.3	Attribute constraints	141

5.3.45.4	Notifications	141

5.3.46	EP_NLG	141

5.3.46.1	Definition	141

5.3.46.2	Attributes	141

5.3.46.3	Attribute constraints	142

5.3.46.4	Notifications	142

5.3.47	EP_N27	142

5.3.47.1	Definition	142

5.3.47.2	Attributes	142

5.3.47.3	Attribute constraints	142

5.3.47.4	Notifications	142

5.3.48	EP_N31	142

5.3.48.1	Definition	142

5.3.48.2	Attributes	142

5.3.48.3	Attribute constraints	143

5.3.48.4	Notifications	143

5.3.49	ExternalNRFFunction	143

5.3.49.1	Definition	143

5.3.49.2	Attributes	143

5.3.49.3	Attribute constraints	143

5.3.49.4	Notifications	143

5.3.50	ExternalNSSFFunction	143

5.3.50.1	Definition	143

5.3.50.2	Attributes	143

5.3.50.3	Attribute constraints	144

5.3.50.4	Notifications	144

5.3.51	AMFSet	144

5.3.51.1	Definition	144

5.3.51.2	Attributes	144

5.3.51.3	Attribute constraints	144

5.3.51.4	Notifications	144

5.3.52	AMFRegion	144

5.3.52.1	Definition	144

5.3.52.2	Attributes	145

5.3.52.3	Attribute constraints	145

5.3.52.4	Notifications	145

5.3.53	ExternalAMFFunction	145

5.3.53.1	Definition	145

5.3.53.2	Attributes	145

5.3.53.3	Attribute constraints	145

5.3.53.4	Notifications	145

5.3.54	ManagedNFProfile <<dataType>>	146

5.3.54.1	Definition	146

5.3.54.2	Attributes	146

5.3.54.3	Attribute constraints	146

5.3.54.4	Notifications	146

5.3.55	HostAddr <<choice>>	146

5.3.55.1	Definition	146

5.3.56	NFInfo <<choice>>	147

5.3.56.1	Definition	147

5.3.57	UdmInfo <<dataType>>	147

5.3.57.1	Definition	147

5.3.57.2	Attributes	147

5.3.57.3	Attribute constraints	147

5.3.57.4	Notifications	147

5.3.58	AusfInfo <<dataType>>	147

5.3.58.1	Definition	147

5.3.58.2	Attributes	148

5.3.58.3	Attribute constraints	148

5.3.58.4	Notifications	148

5.3.59	UpfInfo <<dataType>>	148

5.3.59.1	Definition	148

5.3.59.2	Attributes	148

5.3.59.3	Attribute constraints	148

5.3.59.4	Notifications	148

5.3.60	Void	148

5.3.61	Udrinfo <<dataType>>	148

5.3.61.1	Definition	148

5.3.61.2	Attributes	149

5.3.61.3	Attribute constraints	149

5.3.61.4	Notifications	149

5.3.62	EP_N32	149

5.3.62.1	Definition	149

5.3.62.2	Attributes	149

5.3.62.3	Attribute constraints	149

5.3.62.4	Notifications	149

5.3.63	ExternalSEPPFunction	149

5.3.63.1	Definition	149

5.3.63.2	Attributes	150

5.3.63.3	Attribute constraints	150

5.3.63.4	Notifications	150

5.3.64	SEPPFunction <<ProxyClass>>	150

5.3.64.1	Definition	150

5.3.64.2	Attributes	150

5.3.64.3	Attribute constraints	150

5.3.64.4	Notifications	150

5.3.65	NEFFunction	150

5.3.65.1	Definition	150

5.3.65.2	Attributes	150

5.3.65.3	Attribute constraints	151

5.3.65.4	Notifications	151

5.3.66	SCPFunction	151

5.3.67.1	Definition	151

5.3.67.2	Attributes	151

5.3.67.3	Attribute constraints	151

5.3.67.4	Notifications	151

5.3.68	SupportedFunction <<dataType>>	151

5.3.68.1	Definition	151

5.3.68.2	Attributes	152

5.3.68.3	Attribute constraints	152

5.3.68.4	Notifications	152

5.3.69	CommModel <<dataType>>	152

5.3.69.1	Definition	152

5.3.69.2	Attributes	152

5.3.69.3	Attribute constraints	152

5.3.69.4	Notifications	152

5.3.70	QFQoSMonitoringControl	152

5.3.70.1	Definition	152

5.3.70.2	Attributes	153

5.3.70.3	Attribute constraints	153

5.3.70.4	Notifications	153

5.3.71	QFDelayThresholdsType <<dataType>>	153

5.3.71.1	Definition	153

5.3.71.2	Attributes	153

5.3.71.3	Attribute constraints	153

5.3.71.4	Notifications	154

5.3.72	GtpUPathQoSMonitoringControl	154

5.3.72.1	Definition	154

5.3.72.2	Attributes	154

5.3.72.3	Attribute constraints	154

5.3.72.4	Notifications	154

5.3.73	GtpUPathDelayThresholdsType <<dataType>>	154

5.3.73.1	Definition	154

5.3.73.2	Attributes	155

5.3.73.3	Attribute constraints	155

5.3.73.4	Notifications	155

5.3.75	Configurable5QISet	155

5.3.75.1	Definition	155

5.3.75.2	Attributes	155

5.3.75.3	Attribute constraints	155

5.3.75.4	Notifications	155

5.3.76	FiveQICharacteristics	155

5.3.76.1	Definition	155

5.3.76.2	Attributes	156

5.3.76.3	Attribute constraints	156

5.3.76.4	Notifications	156

5.3.77	PacketErrorRate <<dataType>>	156

5.3.77.1	Definition	156

5.3.77.2	Attributes	156

5.3.77.3	Attribute constraints	156

5.3.77.4	Notifications	156

5.3.78	FiveQiDscpMappingSet	156

5.3.78.1	Definition	156

5.3.78.2	Attributes	157

5.3.78.3	Attribute constraints	157

5.3.78.4	Notifications	157

5.3.79	FiveQiDscpMapping <<dataType>>	157

5.3.79.1	Definition	157

5.3.79.2	Attributes	157

5.3.79.3	Attribute constraints	157

5.3.79.4	Notifications	157

5.3.80	PredefinedPccRuleSet	157

5.3.80.1	Definition	157

5.3.80.2	Attributes	157

5.3.80.3	Attribute constraints	158

5.3.80.4	Notifications	158

5.3.81	PccRule <<dataType>>	158

5.3.81.1	Definition	158

5.3.81.2	Attributes	158

5.3.81.3	Attribute constraints	158

5.3.81.4	Notifications	158

5.3.82	FlowInformation <<dataType>>	159

5.3.82.1	Definition	159

5.3.82.2	Attributes	159

5.3.82.3	Attribute constraints	159

5.3.82.4	Notifications	159

5.3.83	EthFlowDescription <<dataType>>	159

5.3.83.1	Definition	159

5.3.83.2	Attributes	159

5.3.83.3	Attribute constraints	160

5.3.83.4	Notifications	160

5.3.84	QoSData <<dataType>>	160

5.3.84.1	Definition	160

5.3.84.2	Attributes	160

5.3.84.3	Attribute constraints	160

5.3.84.4	Notifications	160

5.3.85	ARP <<dataType>>	160

5.3.85.1	Definition	160

5.3.85.2	Attributes	161

5.3.85.3	Attribute constraints	161

5.3.85.4	Notifications	161

5.3.86	TrafficControlData <<dataType>>	161

5.3.86.1	Definition	161

5.3.86.2	Attributes	161

5.3.86.3	Attribute constraints	161

5.3.86.4	Notifications	161

5.3.87	RedirectInformation <<dataType>>	162

5.3.87.1	Definition	162

5.3.87.2	Attributes	162

5.3.87.3	Attribute constraints	162

5.3.87.4	Notifications	162

5.3.88	RouteToLocation <<dataType>>	162

5.3.88.1	Definition	162

5.3.88.2	Attributes	162

5.3.88.3	Attribute constraints	162

5.3.88.4	Notifications	162

5.3.89	RouteInformation <<dataType>>	163

5.3.89.1	Definition	163

5.3.89.2	Attributes	163

5.3.89.3	Attribute constraints	163

5.3.89.4	Notifications	163

5.3.90	UpPathChgEvent <<dataType>>	163

5.3.90.1	Definition	163

5.3.90.2	Attributes	163

5.3.90.3	Attribute constraints	163

5.3.90.4	Notifications	163

5.3.91	SteeringMode <<dataType>>	164

5.3.91.1	Definition	164

5.3.91.2	Attributes	164

5.3.91.3	Attribute constraints	164

5.3.91.4	Notifications	164

5.3.92	ConditionData  <<dataType>>	164

5.3.92.1	Definition	164

5.3.92.2	Attributes	164

5.3.92.3	Attribute constraints	164

5.3.92.4	Notifications	165

5.3.93	TscaiInputContainer  <<dataType>>	165

5.3.93.1	Definition	165

5.3.93.2	Attributes	165

5.3.93.3	Attribute constraints	165

5.3.93.4	Notifications	165

5.3.94	Dynamic5QISet	165

5.3.94.1	Definition	165

5.3.94.2	Attributes	165

5.3.94.3	Attribute constraints	165

5.3.94.4	Notifications	165

5.3.95	NetworkSliceInfo <<dataType>>	165

5.3.95.1	Definition	165

5.3.95.2	Attributes	166

5.3.95.3	Attribute constraints	166

5.3.95.4	Notifications	166

5.3.96	NSACFFunction	166

5.3.96.1	Definition	166

5.3.96.2	Attributes	166

5.3.96.3	Attribute constraints	166

5.3.96.4	Notifications	166

5.3.97	SnssaiInfo <<dataType>>	166

5.3.97.1	Definition	166

5.3.97.2	Attributes	167

5.3.97.3	Attribute constraints	167

5.3.97.4	Notifications	167

5.3.98	NsacfInfoSnssai <<dataType>>	167

5.3.98.1	Definition	167

5.3.98.2	Attributes	167

5.3.98.3	Attribute constraints	167

5.3.98.4	Notifications	167

5.3.99	EP_N60	167

5.3.99.1	Definition	167

5.3.99.2	Attributes	168

5.3.99.3	Attribute constraints	168

5.3.99.4	Notifications	168

5.3.100	EP_N33	168

5.3.100.1	Definition	168

5.3.100.3	Attributes	168

5.3.100.3	Attribute constraints	168

5.3.100.4	Notifications	168

5.3.101	DDNMFFunction	168

5.3.101.1	Definition	168

5.3.101.2	Attributes	168

5.3.102	EP_N64	169

5.3.102.1	Definition	169

5.3.102.2	Attributes	169

5.3.102.3	Attribute constraints	169

5.3.102.4	Notifications	169

5.3.103	EP_N65	169

5.3.103.1	Definition	169

5.3.103.2	Attributes	169

5.3.103.3	Attribute constraints	170

5.3.103.4	Notifications	170

5.3.104	EP_N66	170

5.3.104.1	Definition	170

5.3.104.2	Attributes	170

5.3.104.3	Attribute constraints	170

5.3.104.4	Notifications	170

5.3.105	GUAMInfo <<dataType>>	170

5.3.105.1	Definition	170

5.3.105.2	Attributes	170

5.3.105.3	Notifications	170

5.3.106	TaiRange <<dataType>>	170

5.3. 106.1	Definition	170

5.3.106.2	Attributes	171

5.3.106.3	Notifications	171

5.3.107	nRTACRange <<dataType>>	171

5.3.107.1	Definition	171

5.3.107.2	Attributes	171

Either the start and end attributes, or the pattern attribute, shall be present.	171

5.3.107.3	Notifications	171

5.3.108	SCPInfo <<dataType>>	171

5.3.108.1	Definition	171

5.3.108.2	Attributes	171

5.3.108.3	Notifications	171

5.3.109	SCPDomainInfo <<dataType>>	172

5.3.109.1	Definition	172

5.3.109.2	Attributes	172

5.3.109.3	Notifications	172

5.3.110	IpEndPoint <<dataType>>	172

5.3.110.1	Definition	172

5.3.110.2	Attributes	172

5.3.110.3	Notifications	172

5.3.111	IPv4AddressRange <<dataType>>	172

5.3.111.1	Definition	172

5.3.111.2	Attributes	173

5.3.111.3	Notifications	173

5.3.112	IPv6PrefixRange <<dataType>>	173

5.3.112.1	Definition	173

5.3.112.2	Attributes	173

5.3.112.3	Notifications	173

5.4	Attribute definitions	174

5.4.1	Attribute properties	174

5.5	Common notifications	198

5.5.1	Alarm notifications	198

5.5.2	Configuration notifications	198

5.5.3	Threshold Crossing notifications	198

5A	Information model definitions for SBA support of IMS	199

5A.1	Imported information entities and local labels	199

5A.2	Class diagram	199

5A.2.1	Class diagram for SBA support of IMS	199

5A.2.1.1	 Relationships	199

5A.2.1.2	Inheritance	200

5A.3	Class definitions	200

5A.3.1	EP_N5	200

5A.3.1.1	Definition	200

5A.3.1.2	Attributes	200

5A.3.2	EP_N70	201

5A.3.2.1	Definition	201

5A.3.2.2	Attributes	201

5A.3.3	EP_N71	201

5A.3.3.1	Definition	201

5A.3.3.2	Attributes	201

6	Information model definitions for network slice NRM	201

6.1	Imported information entities and local labels	201

6.2	Class diagram	202

6.2.1	Relationships	202

6.2.2	Inheritance	203

6.3	Class definitions	203

6.3.1	NetworkSlice	203

6.3.1.1	Definition	203

6.3.1.2	Attributes	203

6.3.1.3	Attribute constraints	204

6.3.1.4	Notifications	204

6.3.2	NetworkSliceSubnet	204

6.3.2.1	Definition	204

6.3.2.2	Attributes	204

6.3.2.3	Attribute constraints	204

6.3.2.4	Notifications	204

6.3.3	ServiceProfile <<dataType>>	205

6.3.3.1	Definition	205

6.3.3.2	Attributes	205

6.3.3.3	Attribute constraints	205

6.3.3.4	Notifications	206

6.3.4	SliceProfile <<dataType>>	206

6.3.4.1	Definition	206

6.3.4.2	Attributes	206

6.3.4.3	Attribute constraints	206

6.3.4.4	Notifications	207

6.3.5	NsInfo <<dataType>>	207

6.3.5.1	Definition	207

6.3.5.2	Attributes	207

6.3.5.3	Attribute constraints	207

6.3.5.4	Notifications	207

6.3.6	ServAttrCom <<dataType>>	207

6.3.6.1	Definition	207

6.3.6.2	Attributes	207

6.3.6.3	Attribute constraints	207

6.3.6.4	Notifications	208

6.3.7	DelayTolerance<<dataType>>	208

6.3.7.1	Definition	208

6.3.7.2	Attributes	208

6.3.7.3	Attribute constraints	208

6.3.7.4	Notifications	208

6.3.8	DeterminComm <<dataType>>	208

6.3.8.1	Definition	208

6.3.8.2	Attributes	208

6.3.8.3	Attribute constraints	208

6.3.8.4	Notifications	208

6.3.9	XLThpt<<dataType>>	209

6.3.9.1	Definition	209

6.3.9.2	Attributes	209

6.3.9.3	Attribute constraints	209

6.3.9.4	Notifications	209

6.3.10	Void	209

6.3.11	MaxPktSize <<dataType>>	209

6.3.11.1	Definition	209

6.3.11.2	Attributes	209

6.3.11.3	Attribute constraints	209

6.3.11.4	Notifications	210

6.3.12	MaxNumberofPDUSessions <<dataType>>	210

6.3.12.1	Definition	210

6.3.12.2	Attributes	210

6.3.12.3	Attribute constraints	210

6.3.12.4	Notifications	210

6.3.13	Void	210

6.3.14	KPIMonitoring <<dataType>>	210

6.3.14.1	Definition	210

6.3.14.2	Attributes	210

6.3.14.3	Attribute constraints	210

6.3.14.4	Notifications	210

6.3.15	UserMgmtOpen<<dataType>>	211

6.3.15.1	Definition	211

6.3.15.2	Attributes	211

6.3.15.3	Attribute constraints	211

6.3.15.4	Notifications	211

6.3.16	V2XCommMode<<dataType>>	211

6.3.16.1	Definition	211

6.3.16.2	Attributes	211

6.3.16.3	Attribute constraints	211

6.3.16.4	Notifications	211

6.3.17	TermDensity<<dataType>>	211

6.3.17.1	Definition	211

6.3.17.2	Attributes	212

6.3.17.3	Attribute constraints	212

6.3.17.4	Notifications	212

6.3.18	EP_Transport	212

6.3.18.1	Definition	212

6.3.18.2	Attributes	212

6.3.18.3	Attribute constraints	212

6.3.18.4	Notifications	212

6.3.19	EP_Application <<ProxyClass>>	213

6.3.19.1	Definition	213

6.3.19.2	Attributes	213

6.3.19.3	Attribute constraints	213

6.3.19.4	Notifications	213

6.3.20	NBIoT <<dataType>>	213

6.3.20.1	Definition	213

6.3.20.2	Attributes	213

6.3.20.3	Attribute constraints	213

6.3.20.4	Notifications	213

6.3.21	Void	213

6.3.22	Void	213

6.3.23	CNSliceSubnetProfile<<dataType>>	213

6.3.23.1	Definition	213

6.3.23.2	Attributes	214

6.3.23.3	Attribute constraints	214

6.3.23.4	Notifications	214

6.3.24	RANSliceSubnetProfile<<dataType>>	214

6.3.24.1	Definition	214

6.3.24.2	Attributes	215

6.3.24.3	Attribute constraints	215

6.3.24.4	Notifications	215

6.3.25	TopSliceSubnetProfile<<dataType>>	215

6.3.25.1	Definition	215

6.3.25.2	Attributes	216

6.3.25.3	Attribute constraints	216

6.3.25.4	Notifications	216

6.3.26	Positioning <<dataType>>	216

6.3.26.1	Definition	216

6.3.26.2	Attributes	216

6.3.26.3	Attribute constraints	217

6.3.26.4	Notifications	217

6.3.27	Synchronicity <<dataType>>	217

6.3.27.1	Definition	217

6.3.27.2	Attributes	217

6.3.27.3	Attribute constraints	217

6.3.27.4	Notifications	217

6.3.28	PositioningRANSubnet <<dataType>>	217

6.3.28.1	Definition	217

6.3.28.2	Attributes	217

6.3.28.3	Attribute constraints	217

6.3.28.4	Notifications	217

6.3.29	SynchronicityRANSubnet <<dataType>>	218

6.3.29.1	Definition	218

6.3.29.2	Attributes	218

6.3.29.3	Attribute constraints	218

6.3.29.4	Notifications	218

6.3.30	EnergyEfficiency <<dataType>>	218

6.3.30.1	Definition	218

6.3.30.2	Attributes	218

6.3.30.3	Attribute constraints	218

6.3.30.4	Notifications	218

6.3.31	RadioSpectrum <<dataType>>	218

6.3.31.1	Definition	218

6.3.31.2	Attributes	219

6.3.31.3	Attribute constraints	219

6.3.31.4	Notifications	219

6.4	Attribute definition	220

6.4.1	Attribute properties	220

6.5	Common notifications	232

6.5.1	Alarm notifications	232

6.5.2	Configuration notifications	232

6.5.3	Threshold Crossing notifications	232

7	Solution Set (SS)	232

Annex A (normative): Cell state handling	241

A.1	Relation between the administrative state and the "Pre-operation state of the gNB-DU Cell"	241

A.2	Combined state diagram for gNB cell	241

Annex B (normative): NSI and NSSI state handling	244

B.1	NSI state handling	244

B.2	State handling of NSSI	245

Annex C (normative): Void	247

Annex D (normative): OpenAPI definition of the NR NRM	248

D.1	General	248

D.2	Void	248

D.3	Void	248

D.4	Solution Set (SS) definitions	248

D.4.1	Void	248

D.4.2	Void	248

D.4.3	OpenAPI document "nrNrm.yaml"	248

Annex E (normative): YANG definitions for NR NRM	274

E.1	General	274

E.2	Void	274

E.3	Void	274

E.4	Void	274

E.5	Modules	274

E.5.1	module _3gpp-nr-nrm-beam.yang	274

E.5.1a	module _3gpp-nr-nrm-bwp.yang	276

E.5.1b	module _3gpp-nr-nrm-commonbeamformingfunction@2019-11-22.yang	277

E.5.2	module _3gpp-nr-nrm-ep.yang	278

E.5.3	module _3gpp-nr-nrm-eutrancellrelation@2019-10-28.yang	281

E.5.4	module _3gpp-nr-nrm-eutranetwork@2019-06-17.yang	284

E.5.5	module _3gpp-nr-nrm-eutranfreqrelation@2019-10-28.yang	284

E.5.6	module _3gpp-nr-nrm-eutranfrequency@2019-10-28.yang	287

E.5.7	module _3gpp-nr-nrm-externalamffunction@2019-10-28.yang	288

E.5.8	module _3gpp-nr-nrm-externalenbfunction@2019-10-28.yang	289

E.5.9	module _3gpp-nr-nrm-externaleutrancell@2019-10-28.yang	290

E.5.10	module _3gpp-nr-nrm-externalgnbcucpfunction@2019-10-28.yang	292

E.5.11	module _3gpp-nr-nrm-externalgnbcuupfunction@2019-10-28.yang	293

E.5.12	module _3gpp-nr-nrm-externalgnbdufunction@2019-10-28.yang	294

E.5.13	module _3gpp-nr-nrm-externalnrcellcu@2019-10-28.yang	295

E.5.14	module _3gpp-nr-nrm-externalservinggwfunction@2019-10-28.yang	296

E.5.15	module _3gpp-nr-nrm-externalupffunction@2019-10-28.yang	297

E.5.16	module _3gpp-nr-nrm-gnbcucpfunction.yang	297

E.5.17	module _3gpp-nr-nrm-gnbcuupfunction.yang	299

E.5.18	module _3gpp-nr-nrm-gnbdufunction.yang	301

E.5.19	module _3gpp-nr-nrm-nrcellcu.yang	303

E.5.20	module _3gpp-nr-nrm-nrcelldu.yang	304

E.5.21	module _3gpp-nr-nrm-nrcellrelation.yang	307

E.5.22	module _3gpp-nr-nrm-nrfreqrelation@2019-10-28.yang	309

E.5.23	module _3gpp-nr-nrm-nrfrequency@2019-10-28.yang	312

E.5.24	module _3gpp-nr-nrm-nrnetwork@2019-06-17.yang	313

E.5.25	module _3gpp-nr-nrm-nrsectorcarrier.yang	314

E.5.26	module _3gpp-nr-nrm-rrmpolicy.yang	315

E.5.27	Void	317

E.5.28	module _3gpp-nr-nrm-danrmanagementfunction.yang	317

E.5.29	module _3gpp-nr-nrm-desmanagementfunction.yang	318

E.5.30	module _3gpp-nr-nrm-drachoptimizationfunction.yang	320

E.5.31	module _3gpp-nr-nrm-dmrofunction.yang	322

E.5.32	module _3gpp-nr-nrm-dpciconfigurationfunction.yang	323

E.5.33	module _3gpp-nr-nrm-cpciconfigurationfunction.yang	324

E.5.34	module _3gpp-nr-nrm-cesmanagementfunction.yang	325

E.6	Void	329

E.7	Mount information	329

Annex F (normative): Void	330

Annex G (normative): OpenAPI definition of the 5GC NRM	331

G.1	General	331

G.2	Void	331

G.3	Void	331

G.4	Solution Set (SS) definitions	331

G.4.1	Void	331

G.4.2	Void	331

G.4.3	OpenAPI document "5gcNrm.yaml"	331

Annex H (normative): YANG definitions for 5GC	373

H.1	General	373

H.2	Void	373

H.3	Void	373

H.4	Void	373

H.5	Modules	373

H.5.1	module _3gpp-5gc-common-yang-types.yang	373

H.5.1a	module _3gpp-5gc-nrm-affunction@2019-10-28.yang	375

H.5.2	module _3gpp-5gc-nrm-amffunction.yang	376

H.5.3	module _3gpp-5gc-nrm-amfregion.yang	377

H.5.4	module _3gpp-5gc-nrm-amfset.yang	378

H.5.5	module _3gpp-5gc-nrm-ausffunction.yang	379

H.5.6	module _3gpp-5gc-nrm-dnfunction@2019-10-28.yang	380

H.5.7	module _3gpp-5gc-nrm-ep@2019-11-18.yang	381

H.5.8	module _3gpp-5gc-nrm-externalnrffunction@2019-10-28.yang	389

H.5.9	module _3gpp-5gc-nrm-externalnssffunction@2019-10-28.yang	390

H.5.10	module _3gpp-5gc-nrm-lmffunction@2019-10-25.yang	390

H.5.11	module _3gpp-5gc-nrm-n3iwffunction@2019-10-28.yang	391

H.5.12	module _3gpp-5gc-nrm-nfprofile@2019-06-17.yang	392

H.5.13	module _3gpp-5gc-nrm-nfservice.yang	407

H.5.14	module _3gpp-5gc-nrm-ngeirfunction.yang	411

H.5.15	module _3gpp-5gc-nrm-nrffunction.yang	412

H.5.16	module _3gpp-5gc-nrm-nssffunction.yang	413

H.5.17	module _3gpp-5gc-nrm-nwdaffunction.yang	414

H.5.18	module _3gpp-5gc-nrm-pcffunction.yang	416

H.5.19	module _3gpp-5gc-nrm-seppfunction.yang	417

H.5.19a	 module _3gpp-5gc-nrm- externalseppfunction@2019-11-17.yang	418

H.5.20	module _3gpp-5gc-nrm-smffunction.yang	419

H.5.21	module _3gpp-5gc-nrm-smsffunction@2019-10-25.yang	420

H.5.22	module _3gpp-5gc-nrm-udmfunction.yang	421

H.5.23	module _3gpp-5gc-nrm-udrfunction.yang	422

H.5.24	module _3gpp-5gc-nrm-udsffunction.yang	423

H.5.25	module _3gpp-5gc-nrm-upffunction.yang	424

H.5.26	module _3gpp-5gc-nrm-scpfunction.yang	425

H.5.27	module _3gpp-5gc-nrm-neffunction.yang	426

H.5.28	module _3gpp-5gc-nrm-QFQoSMonitoringControl.yang	427

H.5.29	module _3gpp-5gc-nrm-GtpUPathQoSMonitoringControl.yang	429

H.5.30	module _3gpp-5gc-nrm-Configurable5QISet.yang	431

H.5.31	module _3gpp-5gc-nrm-FiveQiDscpMappingSet.yang	433

H.5.32	module _3gpp-5gc-nrm-PredefinedPccRuleSet.yang	433

H.5.33	module _3gpp-5gc-nrm-dynamic5QISet.yang	442

H.6	Void	443

H.7	Mount information	443

Annex I (normative): Void	444

Annex J (normative): OpenAPI definition of the Slice NRM	445

J.1	General	445

J.2	Void	445

J.3	Void	445

J.4	Solution Set (SS) definitions	445

J.4.1	Void	445

J.4.2	Void	445

J.4.3	OpenAPI document "sliceNrm.yaml"	445

Annex K (normative): Void	454

Annex L (normative): Relation of GSMA GST, ServiceProfile and SliceProfile	455

L.1	General	455

L.2	GSMA GST, ServiceProfile and sliceProfile	455

Annex M (normative): Managed NF Service state handling	457

M.1	Combined state diagram for a Managed NF Service	457

Annex N (normative): YANG definition of the Slice NRM	459

N.1	General	459

N.2	Modules	459

N.2.1	module _3gpp-ns-nrm-networkslice.yang	459

N.2.2	module _3gpp-ns-nrm-networkslicesubnet.yang	460

N.2.3	Void	463

N.2.4	module _3gpp-ns-nrm-serviceprofile.yang	463

N.2.5	module _3gpp-ns-nrm-sliceprofile.yang	469

N.2.6	module _3gpp-ns-nrm-common.yang	483

Annex O (informative): Change history	485

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

The present document is part of a TS-family covering the 3rd Generation Partnership Project Technical Specification Group Services and System Aspects Management and orchestration of networks, as identified below:

TS 28.540:	Management and orchestration of 5G networks; Network Resource Model (NRM); Stage 1.

TS 28.541:	Management and orchestration of 5G networks; Network Resource Model (NRM); Stage 2 and stage 3.

# 1	Scope

The present document specifies the Information Model and Solution Set for the Network Resource Model (NRM) definitions of NR, NG-RAN, 5G Core Network (5GC) and network slice, to fulfil the requirements identified in TS 28.540 [10].

The Information Model defines the semantics and behaviour of information object class attributes and relations visible on the management interfaces in a protocol and technology neutral way. And Solution Set defines one or more solution set(s) with specific protocol(s) according to the Information Model definitions.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System".

[3]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[4]	3GPP TS 38.401: "NG-RAN; Architecture description".

[5]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[6]	3GPP TS 38.420: "NG-RAN; Xn general aspects and principles".

[7]	3GPP TS 38.470: "NG-RAN; F1 general aspects and principles".

[8]	3GPP TS 38.473: "NG-RAN; F1 application protocol (F1AP)".

[9]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage 2".

[10]	3GPP TS 28.540: "Management and orchestration; 5G Network Resource Model (NRM);Stage 1".

[11]	3GPP TS 28.662: "Telecommunication management; Generic Radio Access Network (RAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS) ".

[12]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[13]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[14]	3GPP TS 36.410: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 general aspects and principles".

[15]	3GPP TS 36.423: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 application protocol".

[16]	3GPP TS 36.425: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 interface user plane protocol".

[17]	3GPP TS 28.625: "State Management Data Definition Integration Reference Point (IRP); Information Service (IS)".

[18]	ITU-T Recommendation X.731: "Information technology - Open Systems Interconnection - Systems Management: State management function".

[19]	3GPP TS 28.658: "Telecommunications management; Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)".

[20]	3GPP TS 28.702: "Core Network (CN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[21]	3GPP TS 28.708: "Telecommunication management; Evolved Packet Core (EPC) Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)".

[22]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[23]	3GPP TS 29.510: "5G system; Network Function Repository Services; Stage 3".

[24]	3GPP TS 29.531: "5G System; Network Slice Selection Services Stage 3".

[25]	Void.

[26]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[27]	3GPP TS 28.554: "Management and orchestration; 5G End to end Key Performance Indicators (KPI)".

[28]	3GPP TS 22.261: "Service requirements for next generation new services and markets".

[29]	ETSI GS NFV-IFA 013 V2.4.1 (2018-02) "Network Function Virtualisation (NFV); Management and Orchestration; Os-Ma-nfvo Reference Point - Interface and Information Model Specification".

[30]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[31]	Void.

[32]	3GPP TS 38.211: "NR; Physical channels and modulation".

[33]	3GPP TS 32.616: "Telecommunication management; Configuration Management (CM); Bulk CM Integration Reference Point (IRP); Solution Set (SS) definitions".

[34]	3GPP TS 28.623: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

[35]	3GPP TS 28.532: "Management and orchestration; Management services".

[36]	Void.

[37]	IETF RFC 791: "Internet Protocol".

[38]	IETF RFC 2373: "IP Version 6 Addressing Architecture".

[39]	IEEE 802.1Q: "Media Access Control Bridges and Virtual Bridged Local Area Networks".

[40]	ETSI GR NFV-IFA 015 (V2.4.1): "Network Function Virtualisation (NFV) Release 2; Management and Orchestration; Report on NFV Information Model".

[41]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[42]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[43]	3GPP TS 32.156: "Telecommunication management; Fixed Mobile Convergence (FMC) model repertoire".

[44]	IETF RFC 4122: "A Universally Unique IDentifier (UUID) URN Namespace".

[45]	IETF RFC 8528: "YANG Schema Mount".

[46]	Void

[47]	3GPP TS 32.160: "Management and orchestration; Management Service Template".

[48]	3GPP TS 38.463: "NG-RAN; E1 application protocol (E1AP)".

[49]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in Idle mode and RRC Inactive state".

[50]	GSMA NG.116 - Generic Network Slice Template Version 3.0 (2020-05-22).

[51]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains; Stage 1".

[52]	3GPP TS 33.501: "Security architecture and procedures for the 5G System".

[53]	3GPP TS 38.901: "Study on channel model for frequencies from 0.5 to 100 GHz ".

[54]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[55]	3GPP TS 38.215: "NR; Physical layer measurements".

[56]	3GPP TS 29.244: "Technical Specification Group Core Network and Terminals; Interface between the Control Plane and the User Plane Nodes; Stage 3".

[57]	3GPP TS 28.313: "Self-Organizing Networks (SON) for 5G networks".

[58]	3GPP TS 38.423: "NR; Xn application protocol (XnAP)".

[59]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[60]	3GPP TS 29.512: "5G System; Session Management Policy Control Service; Stage 3".

[61]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[62]	3GPP TS 29.214: "Policy and Charging Control over Rx reference point".

[63]	IETF RFC 7042: "IANA Considerations and IETF Protocol and Documentation Usage for IEEE 802 Parameters".

[64]	IEEE 802.3-2015: "IEEE Standard for Ethernet".

[65]	IEEE 802.1Q-2014: "Bridges and Bridged Networks".

[66]	IETF RFC 4301: "Security Architecture for the Internet Protocol".

[67]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[68]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[69]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[70]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[71]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[72]	3GPP TS 28.705: "Telecommunication management; IP Multimedia Subsystem (IMS) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[73]	3GPP TS 23.304: " Proximity based Services (ProSe) in the 5G System".

[74]	IETF RFC 8436: " Update to IANA Registration Procedures for Pool 3 Values in the Differentiated Services Field Codepoints (DSCP) Registry".

[75]	ECMA-262: "ECMAScript® Language Specification", https://www.ecma-international.org/ecma-262/5.1/.

[76]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[77]	IANA: "SMI Network Management Private Enterprise Codes", http://www.iana.org/assignments/enterprise-numbers.

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1], TS 28.540 [10] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1] and TS 28.540 [10].

## 3.2	Symbols

void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], TS 23.501 [2], TS 38.401 [4], TS 28.540 [10] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] , TS 23.501 [2], TS 38.401 [4] and TS 28.540 [10].

BWP	Bandwidth part

CM	Configuration Management

DN	Distinguished Name

IOC	Information Object Class

JSON	JavaScript Object Notation

NFV	Network Functions Virtualisation

NRM	Network Resource Model

NS	Network Service

NSI	Network Slice Instance

NSSAI	Network Slice Selection Assistance Information

NSSI	Network Slice Subnet Instance

PNF	Physical Network Function

RIM	Remote interference management

RIM-RS	Remote interference management reference signal

SBA	Service Based Architecture

SS	Solution Set

TN	Transport Network

VNF	Virtualised Network Function