| 3GPP TR 28.886 V0.2.0 (2025-10) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Management Data Analytics (MDA) phase 4 (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	7

4	Concept and background	8

5	Use cases	8

5.1	Improvements to interoperability	8

5.1.X	Use case X: Use case title	8

5.2	Improvements to MDA framework	8

5.2.1	Use case 1: Integration of analytics with a network operator’s management system	8

5.2.2	Use case 2: Indicating supported domain information in MDAFunction	9

5.3	Investigate new and enhanced analytics related capabilities	10

5.3.1	Use case 1: Enhance the Mobility performance analysis use case	10

5.3.2	Use case 2: MDA assisted failure resolution	11

5.3.3	Use case 3: Radio resource optimization based on per SSB usage	12

5.3.4	Use case 4: Remote Electrical Tilt (RET) and Transmission Power analytics (TP)	13

TBD6	Conclusions and recommendations	14

6.1	Improvements to interoperability	14

6.2	Improvements to MDA framework	14

6.3	New and enhanced analytics capabilities	14

# Foreword

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

# 1	Scope

The present document …

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.104: "Management Data Analytics (MDA)".

[3]	3GPP TS 28.541: "5G Network Resource Model (NRM); Stage 2 and stage 3".

[4]	3GPP TS 28.567: "Management aspects of closed control loops".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

CPA	Conditional PSCell Addition

CPC	Conditional PSCell Change

SN 	Secondary Node

NSA 	Non-standalone

# 4	Concept and background

# 5	Use cases

## 5.1	Improvements to interoperability

### 5.1.X	Use case X: Use case title

#### 5.1.X.1	Description

#### 5.1.X.2	Potential requirements

#### 5.1.X.3	Potential solutions

#### 5.1.X.4	Evaluation of solutions

## 5.2	Improvements to MDA framework

### 5.2.1	Use case 1: Integration of analytics with a network operator’s management system

#### 5.2.1.1	Description

A Network Operator (NOP) wishes to integrate Management Data Analytics Service with the NOP’s automated network management system.

To be able to successfully perform this integration, the NOP needs to know which management facilities are provided by MDAS. The NOP also needs to know if MDAS depends on other 3GPP management functions and/or management services, and if these dependencies are mandatory or optional.

The operator uses TS 28.104 [2] to learn the required information about MDAS. Clause 5 of TS 28.104 [2] describes the management facilities which are provided by MDAS. Clause 5 of TS 28.104 [2] describes how MDAS may be combined with other 3GPP management functions and/or management services to satisfy NOP requirements.

Currently, clause 5 of TS 28.104 [2] is lacking description on how MDAS relates to some 3GPP management functions and/or management services (e.g. CCL, NDT). The current description is very abstract and may benefit from adding examples. The current description may also conflict with the content of other technical specifications.

#### 5.2.1.2	Potential requirements

None.

#### 5.2.1.3	Potential solutions

##### 5.2.1.3.1	Update description of MDA scope for closed loop control management

TS 28.104 [2] clause 6.1 states that only analytics are in scope. Other aspects of a management loop (Observation, Decision, Execution) are therefore not in scope. This is in conflict with TS 28.567 [4] figures 5.1.2.2-1 and 5.1.2.2-2, which show that the MDA MnS producer may perform data collection, analytics, and decision making.

It is proposed to update TS 28.567 [4] figure 5.1.2.2-1, figure 5.1.2.2-2 and associated text to clarify that only analytics are in scope of the MDA MnS producer.

#### 5.2.1.4	Evaluation of solutions

### 5.2.2	Use case 2: Indicating supported domain information in MDAFunction

#### 5.2.2.1	Description

The IOC MDAFunction represents the MDA function which supports one or more MDA capabilities as described in TS 28.104 [2]. It also describes that an MDA function can act as 3GPP domain-specific (e.g. RAN or CN) or as 3GPP cross-domain MDA MnS producer. From MnS consumer perspective, it is reasonable to know not only the support MDA capabilities but also to identify in which domains the MDA Function is supported for corresponding analysis. For example, an MnS consumer may wish to perform analytic for CN, while the MDA producer is only capable to provide report for RAN. There is no way to identify the capability of the MDA producer in advance, and no way to report why the producer cannot satisfy the request.

Currently, in clause 9.3.1.2 in 28.104 [2] the definition of MDAFunction IOC, only contains supportedMDACapabilities attribute without domain information. This makes a MnS consumer difficult to know the actual analysis scope of a MDAFunction.

#### 5.2.2.2	Potential requirements

REQ-MDA_Fun-01: The MDA MnS producer should have the capability to allow authorized MDA MnS consumer to request the supported analysis of domain information.

#### 5.2.3.3	Potential solutions

It is proposed to make the following changes to TS 28.104 [2] clause 9.3.1.2:

- update the MDAFunction IOC attribute to add a new attribute supportedMDADomains, which can be used to indicate the domain(s) supported by the MDAFunction.

#### 5.2.2.4	Evaluation of solutions

## 5.3	Investigate new and enhanced analytics related capabilities

### 5.3.1	Use case 1: Enhance the Mobility performance analysis use case

#### 5.3.1.1	Description

This use case is to enhance the existing MDA capability for mobility performance analysis. The enhancements to the existing mobility performance analysis are targeted towards to solve the mobility problems in NSA deployment architecture and to recommend optimization or repair actions.

The existing use case for mobility performance analysis is described in TS 28.104 [2]. This analysis output includes information such as mobility issue root cause, mobility issue location. The descriptions and requirements in the existing use cases also mention performance analysis under NSA deployment scenarios. However, the current solution does not provide additional information (e.g. mobility issue type such as NSA mobility issue and SA mobility issue, or the root cause of NSA mobility issue, or the recommended actions) which may enable the consumer to anticipate the impacts to the network more effectively.

The mobility performance related problems in NSA deployment scenarios may result from too-early/too-late PSCell change or too-early/too-late conditional PSCell addition or change due to inappropriate handover parameters as defined in TS 38.300 [x]. Performance measurements related to NSA need to be defined and used as enabling data for MDA analysis. MDAS can be used to analyse network performance during handover period in different mobility scenarios. The MDAS producer may also be capable to provide the recommendations of optimal handover parameters to MDAS consumer.

Correspondingly, the root cause of mobility issue in NSA deployment scenarios may be of interest to the consumer and can be provided in the analytics report. The root cause can be high interference in both uplink and downlink, wireless resource issues (such as high utilization of uplink and downlink PRBs), transport layer reasons (including insufficient transport resources leading to SN addition failure, such as S1 interface transport, X2 interface transport), and UE capability issues.

#### 5.3.1.2	Potential requirements

REQ-MRO_MDA-01: MDA capability for mobility performance issue analysis should provide the NSA mobility issue including too-early PSCell change, too-late PSCell change, too Late CPC Execution and too Early CPC/CPA Execution.

REQ-MRO_MDA-02: MDA capability for mobility performance issue analysis should include providing recommended actions to solve the mobility performance issue.

#### 5.3.1.3	Potential solutions

Reuse the performanceThresholdInfo attribute in the MDARequest IOC with the threshold of mobility related performance measurements for this MDA type, including:

-	Thresholds of mobility related performance measurements

The enabling data for this MDA type would introduce the additional NSA mobility measurements as following:

-	SN addition related performance measurements.

-	SN change related performance measurements.

-	Conditional SN addition related performance measurements

-	Conditional SN change related performance measurements

The analytics output for this MDA type would include the following:

-	The mobility performance issue type including NSA mobility issue and SA mobility issue

-	The mobility performance issue including too-early handovers, too-late handovers and ping-pong handovers, too-early PSCell change, too-late PSCell change, too late CPC Execution and too early CPC/CPA execution.

-	The mobility performance issue root case can be such as high interference, insufficient transport resources, UE capability issues etc.;

-	The recommended actions to resolve the mobility issue (e.g. change the mobility parameters (e.g. CIO) or antenna RF parameters such as downtilt and azimuth etc.).

NOTE: The NSA mobility measurements are subject to be defined in TS 28.552 (FFS).

#### 5.3.1.4	Evaluation of solutions

TBD

### 5.3.2	Use case 2: MDA assisted failure resolution

#### 5.3.2.1	Description

As networks become more complex, it becomes more difficult to quickly resolve network failures, where network performance becomes degraded or lost completely. By looking only at the alarm list, it can be difficult to identify the most serious network failures. Network automation is essential to speed up the identification and resolution of network failures, thereby increasing the stability of the network. This use case enables an MDAS consumer (e.g. an automated trouble ticketing system) to automatically perform the steps needed to resolve a network failure.

The MDAS consumer requests the MDAS producer to identify alarms which may relate to network failures. The MDAS producer analyses alarm data and network data to correlate alarms with deteriorating performance in the network. The MDAS producer analyses the cause of deteriorating performance. The MDAS producer reports possible network failures and the possible cause to the MDAS consumer.

The MDAS consumer requests the MDAS producer to diagnose the cause of a network failure. The MDAS producer performs analysis to locate the likely cause of the network failure (e.g. network function, virtualized resource, physical resource). The MDAS producer reports the possible cause of the network failure to the MDAS consumer.

The MDAS consumer requests the MDAS producer to recommend how to resolve a network failure. The MDAS producer performs analysis to identify which domain is causing the network failure. The MDAS producer calculates possible methods (e.g. re-configurations) to resolve the network failure. The MDAS producer simulates or emulates the effect of the possible methods (e.g. by using NDT) to evaluate if the network failure is resolved. The MDAS producer reports the possible methods to the MDAS consumer.

#### 5.3.2.2	Potential requirements

REQ-FAILURE_RESOLUTION_MDA-01: MDA capability for network failure resolution should be able to provide the analytics output including the probable impact of the network failure.

REQ-FAILURE_RESOLUTION_MDA-02: MDA capability for network failure resolution should be able to provide the analytics output including recommended actions to resolve the network failure.

#### 5.3.2.3	Potential solutions

Update TS 28.104 [2] to add a new MDA type MDAAssistedFaultManagement.FailureAnalysis to allow an MnS consumer to request analytics related to a network failure.

Update TS 28.104 [2] to add a new analytics output for network failure analysis, including (but not limited to) information related to

- related alarm(s) which are likely to be correlated to the network failure

- managed objects (e.g. NFs, cells) that are impacted by the network failure

- metrics or KPIs that are impacted by the network failure

- probable impacted domain and cause of the network failure

- recommended actions to resolve the network failure

- estimated time to resolve the network failure

#### 5.3.2.4	Evaluation of solutions

### 5.3.3	Use case 3: Radio resource optimization based on per SSB usage

#### 5.3.3.1	Description

5G NR adopts massive MIMO technology, where the base station employs directional beams for both UE initial access on common channels and subsequent connection during the RRC_CONNECTED state. Common signals/channels for initial access are transmitted within the Synchronization Signal Block (SSB). SSBs are designed with different patterns, corresponding to the number of SSB beams and their distinct directions.

In Clause 4.3.39 of TS 28.541[3], <<IOC>>CommonBeamformingFunction is defined to represents common beamforming functionality (eg: SSB beams). The CommonBeamformingFunction provides capability to configure the advanced antenna for a sector carrier. The configuration capability is provided by selection of coverageShape, digitalTilt and digitalAzimuth. These attributes represent the wanted coverage area and radiation pattern on a sector carrier related to an antenna transmission point.

For example, the area where a large outdoor stadium holds some competitions is characterized by a high number of users which is challenging to guarantee enough radio resource in this scenario. It is necessary to consider using MDAS to improve radio resource usage. MnS consumer could request MDAS to acquire beam-level management data and recommend coverage shape based on analysis. The beam-level data includes Number of UE related the SSB beam Index (mean), DL PRB Usage per SSB, etc. The recommendation could be used for 3GPP Management System to update the configuration of the target cell's coverage shape on gNodeB.

Figure 5.3.3.1-1 depicts SSB beams covering full cell coverage area. Figure 5.3.3.1-2 depicts SSB beams modifying to meet the UE centralized coverage after updating the coverage shape for target cell of gNodeB using the recommendation by MDAS.

It is anticipated that MDAS can be used to obtain an analysis of the usage of each SSB. This information can then be used to adjust the coverage shape.

| Figure 5.3.3.1-1: Illustration of SSB beams covering full cell coverage area | Figure 5.3.3.1-2: Illustration of SSB beams modifying to meet the UE centralized coverage with MDAS recommendation |
| --- | --- |

#### 5.3.3.2	Potential requirements

REQ-RRO-FUN-01: MDA should have capability for radio resource optimization analysis.

REQ-RRO-FUN-02: MDA capability for radio resource optimization analysis should include the capability of the beam-level radio resource utilisation assessment to indicate if coverage shape configuration of gNodeB is proper based on beam-level management data analytics.

REQ-RRO-FUN-03: MDA capability for radio resource optimization analysis should include the capability to recommend coverage shape configuration for target cell of gNodeB.

#### 5.3.3.3		Potential solutions

TBD

#### 5.3.3.4	Evaluation of solutions

TBD

### 5.3.4	Use case 4: Remote Electrical Tilt (RET) and Transmission Power analytics (TP)

#### 5.3.4.1	Description

Operators establish their own guidelines for deploying antenna RET, both mechanical and electrical, in cells, primarily based on LTE/NR layers and traffic management strategies. Initial RET and power configurations may not be optimal for all cells, necessitating ongoing fine-tuning. The initial RET may not account for changes in subscriber movement or infrastructure developments. Factors influencing optimal RET and power can evolve over time, including:

Subscriber Movement: Dynamic changes in user behavior, such as daily commutes from office to home or weekend outings to recreational areas, can impact network performance requiring the modification in the RET.

Infrastructure Growth: The addition of new cell sites can introduce interference with neighboring cells. Interference arises when signals from the new site overlap with or disrupt existing signals. This would affect performance and can be mitigated with modification in the RET.

Weather Conditions: Atmospheric phenomena like rain, snow, and fog can affect radio wave propagation, leading to signal attenuation or distortion. Tilt modification may minimize the impact.

It is desirable to use MDA analytics to generate optimal RET and power configuration over time. It is essential to follow initial antenna RET deployment with subsequent fine-tuning using MDA analytics. The producer employs a machine learning-based model that leverages strong correlations between performance, fault and configuration data of the network. This ML model analyzes the impacts of coverage and interference in relation to user traffic behavior, including mobility and weather conditions. It generates recommendations for fine-tuning RET and optimizing transmission power per cell.

#### 5.3.4.2	Potential requirements

REQ-RET-DATA-1: MDA capability for RET analysis shall include providing analytics for indicating the recommended change in RET and TP for a specific cell.

#### 5.3.4.3	Possible solutions

TBD

#### 5.3.4.4	Evaluation of solutions

TBD

# 6	Conclusions and recommendations

## 6.1	Improvements to interoperability

## 6.2	Improvements to MDA framework

## 6.3	New and enhanced analytics capabilities

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-08 |  |  |  |  |  | Initial skeleton | 0.0.0 |
| 2025-08 | SA5#162 | S5‑253605 | pCR |  |  | Pseudo-CR on TR 28.886 Add clause structure | 0.1.0 |
| 2025-08 | SA5#162 | S5‑254044 | pCR |  |  | Pseudo-CR on TR 28.886 Enhance the mobility performance analysis use case | 0.1.0 |
| 2025-08 | SA5#162 | S5‑254045 | pCR |  |  | Pseudo-CR on TR 28.886 Add MDA integration use case | 0.1.0 |
| 2025-08 | SA5#162 | S5‑254046 | pCR |  |  | Pseudo-CR on TR 28.886 Add failure resolution use case | 0.1.0 |
| 2025-10 | SA5#163 | S5‑254687 | pCR |  |  | Pseudo-CR on TR 28.886 Add solution for enhance the mobility performance analysis use case | 0.2.0 |
| 2025-10 | SA5#163 | S5‑254684 | pCR |  |  | Pseudo-CR on TR 28.886 Add new use case on domain information for MDAFunction | 0.2.0 |
| 2025-10 | SA5#163 | S5‑254685 | pCR |  |  | Rel-20 pCR TR 28.886 Add new use case on radio resource optimization based on per SSB usage | 0.2.0 |
| 2025-10 | SA5#163 | S5‑254686 | pCR |  |  | Rel-20 pCR 28.886 RET Analytics | 0.2.0 |
| 2025-10 | SA5#163 | S5‑254688 | pCR |  |  | Pseudo-CR on TR 28.886 Add requirements and solution for failure resolution | 0.2.0 |
| 2025-10 | SA5#163 | S5‑254782 | pCR |  |  | Pseudo-CR on TR 28.886 Add solution for MDA scope | 0.2.0 |