| 3GPP TS 29.175 V19.3.0 (2025-09) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; IP Multimedia Subsystem; IP Multimedia Subsystem (IMS) Application Server (AS) Services Stage 3 (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	14

1	Scope	16

2	References	16

3	Definitions, symbols and abbreviations	17

3.1	Definitions	17

3.2	Symbols	17

3.3	Abbreviations	17

4	Overview	17

5	Services offered by the IMS AS	18

5.1	Introduction	18

5.2	Nimsas_SessionEventControl Service	19

5.2.1	Service Description	19

5.2.2	Service Operations	19

5.2.2.1	Introduction	19

5.2.2.1A	Subscribe	20

5.2.2.2	Notify	20

5.2.2.2.1	General	20

5.2.2.2.2	Notification for Session Event	20

5.3	Nimsas_MediaControl Service	21

5.3.1	Service Description	21

5.3.2	Service Operations	21

5.3.2.1	Introduction	21

5.3.2.2	MediaInstruction	21

5.3.2.2.1	General	21

5.3.2.2.2	Media Instruction	21

5.4	Nimsas_ImsSessionManagement Service	22

5.4.1	Service Description	22

5.4.2	Service Operations	22

5.4.2.1	Introduction	22

5.4.2.2	Create Service Operation	23

5.4.2.2.1	General	23

5.4.2.2.2	Creation of a new IMS session	23

5.4.2.3	Update Service Operation	23

5.4.2.3.1	General	23

5.4.2.3.2	Partial updating an existing IMS session	24

5.4.2.3.3	Updating an existing IMS session	24

5.4.2.4	Delete Service Operation	25

5.4.2.4.1	General	25

5.4.2.4.2	Releasing an exisiting IMS Session	25

5.4.2.5	Notify	25

5.4.2.5.1	General	25

5.4.2.5.2	Notification for IMS Session Event	25

5.5	Nimsas_EventExposure Service	26

5.5.1	Service Description	26

5.5.2	Service Operations	26

5.5.2.1	Introduction	26

5.5.2.2	Subscribe	26

5.5.2.2.1	General	26

5.5.2.2.2	Subscription for event notifications	27

5.5.2.2.3	Updating an exisiting subscription	27

5.5.2.2.4	Subscription Partial Update	28

5.5.2.3	Unsubscribe	28

5.5.2.3.1	General	28

5.5.2.3.2	Unsubscribe from notifications	28

5.5.2.3	Notify	29

5.5.2.3.1	General	29

5.5.2.2.2	Notification for Session Event	29

5.5.2.4	SubscriptionInfoRetrieval	29

5.5.2.4.1	General	29

5.5.2.4.2	Subscription Info Retrieval	30

5.6	Nimsas_ImsParameterProvision Service	30

5.6.1	Service Description	30

5.6.2	Service Operations	30

5.6.2.1	Introduction	30

5.6.2.2	Create Service Operation	31

5.6.2.2.1	General	31

5.6.2.2.2	Creation of the Parameter Provisioning Data Entry per AF	31

5.6.2.3	Update Service Operation	31

5.6.2.3.1	General	31

5.6.2.3.2	Updating the Parameter Provisioning Data Entry per AF	31

5.6.2.4	Delete Service Operation	32

5.6.2.4.1	General	32

5.6.2.4.2	Deleting the Parameter Provisioning Data Entry per AF	33

6	API Definitions	33

6.1	Nimsas_SessionEventControl Service API	33

6.1.1	API URI	33

6.1.2	Usage of HTTP	34

6.1.2.1	General	34

6.1.2.2	HTTP standard headers	34

6.1.2.2.1	General	34

6.1.2.2.2	Content type	34

6.1.2.3	HTTP custom headers	34

6.1.3	Resources	34

6.1.3.1	Overview	34

6.1.3.2	Resource: Session Event Subscriptions	35

6.1.3.2.1	Description	35

6.1.3.2.2	Resource Definition	35

6.1.3.2.3	Resource Standard Methods	35

6.1.3.2.4	Resource Custom Operations	35

6.1.4	Custom Operations without associated resources	35

6.1.5	Notifications	35

6.1.5.1	General	35

6.1.5.2	Session Event Notification	36

6.1.5.2.1	Description	36

6.1.5.2.2	Target URI	36

6.1.5.2.3	Standard Methods	36

6.1.6	Data Model	37

6.1.6.1	General	37

6.1.6.2	Structured data types	38

6.1.6.2.1	Introduction	38

6.1.6.2.2	Type: SessionEventNotification	39

6.1.6.2.3	Type: NotificationEvent	40

6.1.6.2.4	Type: SessionInfo	40

6.1.6.2.5	Type: MediaInfo	41

6.1.6.2.6	Type: DcMediaSpec	41

6.1.6.2.7	Type: QosHintInfo	42

6.1.6.3	Simple data types and enumerations	42

6.1.6.3.1	Introduction	42

6.1.6.3.2	Simple data types	42

6.1.6.3.3	Enumeration: EventType	42

6.1.6.3.4	Enumeration: MediaType	43

6.1.6.3.5	Enumeration: SessionCase	43

6.1.6.3.6	Enumeration: EventInitiator	43

6.1.6.3.7	Enumeration: PsDataOffStatus	43

6.1.6.4	Data types describing alternative data types or combinations of data types	43

6.1.6.5	Binary data	43

6.1.7	Error Handling	44

6.1.7.1	General	44

6.1.7.2	Protocol Errors	44

6.1.7.3	Application Errors	44

6.1.8	Feature negotiation	44

6.1.9	Security	44

6.1.10	HTTP redirection	45

6.2	Nimsas_MediaControl Service API	45

6.2.1	API URI	45

6.2.2	Usage of HTTP	45

6.2.2.1	General	45

6.2.2.2	HTTP standard headers	45

6.2.2.2.1	General	45

6.2.2.2.2	Content type	45

6.2.2.3	HTTP custom headers	46

6.2.3	Resources	46

6.2.3.1	Overview	46

6.2.3.2	Resource: Individual call session	46

6.2.3.2.1	Description	46

6.2.3.2.2	Resource Definition	46

6.2.3.2.3	Resource Standard Methods	47

6.2.3.2.4	Resource Custom Operations	47

6.2.4	Custom Operations without associated resources	48

6.2.5	Notifications	48

6.2.6	Data Model	48

6.2.6.1	General	48

6.2.6.2	Structured data types	49

6.2.6.2.1	Introduction	49

6.2.6.2.2	Type: MediaInstructionData	50

6.2.6.2.3	Type: MediaInstructions	51

6.2.6.2.4	Type: DcMediaSpecification	52

6.2.6.2.5	Type: ArMediaSpecification	53

6.2.6.2.6	Type: Mdc2EndpointInfo	53

6.2.6.2.7	Type: AudioVideoReNegotiationInd	54

6.2.6.2.8	Type: InterworkingInstruction	54

6.2.6.3	Simple data types and enumerations	54

6.2.6.3.1	Introduction	54

6.2.6.3.2	Simple data types	54

6.2.6.3.3	Enumeration: MediaInstruction	54

6.2.6.3.4	Enumeration: MediaConnSide	55

6.2.6.3.5	Enumeration: TranscodeMode	55

6.2.6.3.6	Enumeration: SendRecvMode	55

6.2.6.4	Data types describing alternative data types or combinations of data types	55

6.2.6.5	Binary data	56

6.2.7	Error Handling	56

6.2.7.1	General	56

6.2.7.2	Protocol Errors	56

6.2.7.3	Application Errors	56

6.2.8	Feature negotiation	56

6.2.9	Security	56

6.2.10	HTTP redirection	56

6.3	Nimsas_ImsSessionManagement Service API	57

6.3.1	API URI	57

6.3.2	Usage of HTTP	57

6.3.2.1	General	57

6.3.2.2	HTTP standard headers	57

6.3.2.2.1	General	57

6.3.2.2.2	Content type	57

6.3.2.3	HTTP custom headers	58

6.3.3	Resources	58

6.3.3.1	Overview	58

6.3.3.2	Resource: IMS Sessions	58

6.3.3.2.1	Description	58

6.3.3.2.2	Resource Definition	58

6.3.3.2.3	Resource Standard Methods	59

6.3.3.2.4	Resource Custom Operations	60

6.3.3.3	Resource: Individual IMS Session	60

6.3.3.3.1	Description	60

6.3.3.3.2	Resource Definition	60

6.3.3.3.3	Resource Standard Methods	60

6.3.3.3.4	Resource Custom Operations	63

6.3.4	Custom Operations without associated resources	63

6.3.5	Notifications	64

6.3.5.1	General	64

6.3.5.2	IMS Session Event Notification	64

6.3.5.2.1	Description	64

6.3.5.2.2	Target URI	64

6.3.5.2.3	Standard Methods	64

6.3.6	Data Model	65

6.3.6.1	General	65

6.3.6.2	Structured data types	66

6.3.6.2.1	Introduction	66

6.3.6.2.2	Type: ImsSessionInfo	67

6.3.6.2.3	Type: SessionInfoSM	67

6.3.6.2.4	Type: MediaInfoExternal	67

6.3.6.2.5	Type: DcMediaPara	68

6.3.6.2.6	Type: ImsSessionEventNotification	68

6.3.6.2.7	Type: TargetIdInfo	68

6.3.6.3	Simple data types and enumerations	69

6.3.6.3.1	Introduction	69

6.3.6.3.2	Simple data types	69

6.3.6.3.3	Enumeration: DcType	69

6.3.6.3.4	Enumeration: AdcType	69

6.3.6.3.5	Void	69

6.3.6.3.6	Enumeration: ImsSessionEventType	69

6.3.6.4	Data types describing alternative data types or combinations of data types	70

6.3.6.5	Binary data	70

6.3.7	Error Handling	70

6.3.7.1	General	70

6.3.7.2	Protocol Errors	70

6.3.7.3	Application Errors	70

6.3.8	Feature negotiation	70

6.3.9	Security	70

6.3.10	HTTP redirection	71

6.4	Nimsas_ImsEE Service API	71

6.4.1	API URI	71

6.4.2	Usage of HTTP	71

6.4.2.1	General	71

6.4.2.2	HTTP standard headers	72

6.4.2.2.1	General	72

6.4.2.2.2	Content type	72

6.4.2.3	HTTP custom headers	72

6.4.3	Resources	72

6.4.3.1	Overview	72

6.4.3.2	Resource: IMS Event Subscriptions	73

6.4.3.2.1	Description	73

6.4.3.2.2	Resource Definition	73

6.4.3.2.3	Resource Standard Methods	73

6.4.3.2.4	Resource Custom Operations	74

6.4.3.3	Resource: Individual IMS Event Subscription	74

6.4.3.3.1	Description	74

6.4.3.3.2	Resource Definition	74

6.4.3.3.3	Resource Standard Methods	74

6.4.3.3.4	Resource Custom Operations	78

6.4.4	Custom Operations without associated resources	79

6.4.5	Notifications	79

6.4.5.1	General	79

6.4.5.2	Event Notification	79

6.4.5.2.1	Description	79

6.4.5.2.2	Target URI	79

6.4.5.2.3	Standard Methods	79

6.4.6	Data Model	80

6.4.6.1	General	80

6.4.6.2	Structured data types	81

6.4.6.2.1	Introduction	81

6.4.6.2.2	Type: ImsEventsSubscription	82

6.4.6.2.3	Void	82

6.4.6.2.4	Type: ImsEventsNotification	82

6.4.6.2.5	Type: EventNotification	83

6.4.6.2.6	Void	83

6.4.6.2.7	Void	83

6.4.6.2.8	Void	83

6.4.6.2.9	Void	83

6.4.6.2.10	Type: ImsEventsSubscriptionCreated	83

6.4.6.2.11	Void	83

6.4.6.3	Simple data types and enumerations	83

6.4.6.3.1	Introduction	83

6.4.6.3.2	Simple data types	83

6.4.6.4	Data types describing alternative data types or combinations of data types	83

6.4.6.5	Binary data	84

6.4.7	Error Handling	84

6.4.7.1	General	84

6.4.7.2	Protocol Errors	84

6.4.7.3	Application Errors	84

6.4.8	Feature negotiation	84

6.4.9	Security	84

6.4.10	HTTP redirection	85

6.5	Nimsas_ImsParameterProvision Service API	85

6.5.1	API URI	85

6.5.2	Usage of HTTP	85

6.5.2.1	General	85

6.5.2.2	HTTP standard headers	85

6.5.2.2.1	General	85

6.5.2.2.2	Content type	85

6.5.2.3	HTTP custom headers	86

6.5.3	Resources	86

6.5.3.1	Overview	86

6.5.3.2	Resource: ImsParameterProvisioningDataEntry	86

6.5.3.2.1	Description	86

6.5.3.2.2	Resource Definition	86

6.5.3.2.3	Resource Standard Methods	87

6.5.3.2.4	Resource Custom Operations	89

6.5.4	Custom Operations without associated resources	89

6.5.5	Notifications	89

6.5.6	Data Model	89

6.5.6.1	General	89

6.5.6.2	Structured data types	90

6.5.6.2.1	Introduction	90

6.5.6.2.2	Type: ImsPpDataEntry	90

6.5.6.3	Simple data types and enumerations	90

6.5.6.3.1	Introduction	90

6.5.6.3.2	Simple data types	90

6.5.6.4	Data types describing alternative data types or combinations of data types	90

6.5.6.5	Binary data	90

6.5.7	Error Handling	91

6.5.7.1	General	91

6.5.7.2	Protocol Errors	91

6.5.7.3	Application Errors	91

6.5.8	Feature negotiation	91

6.5.9	Security	91

6.5.10	HTTP redirection	91

Annex A (normative): OpenAPI specification	93

A.1	General	93

A.2	Nimsas_SessionEventControl API	93

A.3	Nimsas_MediaControl API	97

A.4	Nimsas_ImsSessionManagement API	102

A.5	Nimsas_ImsEE API	109

A.6	Nimsas_ImsParameterProvision API	116

Annex B (informative): Change history	119

Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document specifies the stage 3 protocol and data model for the Nimsas Service Based Interface. It provides stage 3 protocol definitions and message flows, and specifies the API for each service offered by the IMS AS.

The 5G System stage 2 architecture and procedures are specified in TS 23.501 [2] and TS 23.502 [3].

The IP Multimedia Subsystem (IMS) supporting DC architecture and procedures are specified in TS 23.228 [14].

The Technical Realization of the Service Based Architecture and the Principles and Guidelines for Services Definition are specified in TS 29.500 [4] and TS 29.501 [5].

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	OpenAPI: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[7]	3GPP TR 21.900: "Technical Specification Group working methods".

[8]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[9]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[10]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[11]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[12]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[13]	IETF RFC 7807: "Problem Details for HTTP APIs".

[14]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[15]	3GPP TS 29.562: "5G System; Home Subscriber Server (HSS) Services; Stage 3".

[16]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[17]	3GPP TS 29.176: "IP Multimedia Subsystems (IMS); Media Function (MF) Services; Stage 3".

[18]	3GPP TS 24.610:" Communication HOLD (HOLD) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[19]	3GPP TS 24.186:" IMS Data Channel applications; Protocol specification".

[20]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[21]	3GPP TS 24.604: "Communication Diversion (CDIV) using IP Multimedia (IM); Core Network (CN) subsystem; Protocol specification".

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.228 [14] apply:

-	Bootstrap data channel

-	DC Application Server

-	Rich Call Data (RCD) server

-	Rich Call Data (RCD) server address

-	Rich Call Data (RCD) information

-	Rich Call Data (RCD) URL

-	Rich Call Data (RCD) properties

3.2	Symbols

For the purposes of the present document, the following symbols given in 3GPP TS 23.228 [14] apply:

DC1	Reference Point between an SBI capable IMS AS and DCSF.

DC2	Reference Point between an SBI capable IMS AS and MF.

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AR	Augmented Reality

DC	Data Channel

DCSF	Data Channel Signalling Function

HSS	Home Subscriber Server

MF	Media Function

NEF	Network Exposure Function

RCD	Rich Call Data

4	Overview

The IP Multimedia Subsystem (IMS) architecture supporting the Data Channel services is defined in 3GPP TS 23.228 [14].

The IP Multimedia Subsystem (IMS) Application Server (AS) is enhanced to offer services to e.g. DCSF, HSS, NEF or trusted AF via the Nimsas service-based interface (see 3GPP TS 23.228 [14]) and support the following functionalities:

-	The IMS AS interacts with the DCSF via DC1 for event notifications;

-	The IMS AS receives the data channel control instructions from the DCSF and accordingly interacts with the MF via DC2 for data channel media resource management;

-	The IMS AS receives the data channel management requirement from NEF or trusted AF and interacts with the DCSF and MF for data channel management.

-	The IMS AS interacts with the HSS/NEF for subscriber related events subscription, and notifies the NEF with the subscriber related events which are subscribed, and interacts with the NEF for non-subscriber related events subscription and notification, e.g. subscriptions to any UE;

-	The IMS AS receives the provisioning information (e.g. RCD properties) from NEF or trusted AF and stores in HSS repository data to support the 3rd party user identity.

Figure 4-1 provides the reference model (in service-based interface representation and in reference point representation), with focus on the IMS AS.

Figure 4-1: Reference model – IMS AS

DC1 is the reference point between an SBI capable IMS AS and DCSF.

N92 is the reference point between an SBI capable IMS AS and NEF.

N71 is the reference point between an SBI capable IMS AS and HSS.

5	Services offered by the IMS AS

5.1	Introduction

The IMS AS offers to other NFs the following services:

-	Nimsas_SessionEventControl. This service enables the consumer to be notified about session events when served IMS subscribers take part in IMS sessions.

-	Nimsas_MediaControl. This service enables the consumer to control IMS AS handling of media flow within an IMS session.

-	Nimsas_ImsSessionManagement. This service enables the consumer to request IMS AS to create or release an IMS session or modify the media in a specific IMS session.

-	Nimsas_EventExposure. This service enables the service consumer to subscribe to subscriber-related events (e.g. HSS) and non-subscriber related events (e.g. NEF).

-	Nimsas_ImsParameterProvision. This service enables the consumer to provision the IMS user related information (e.g. RCD URL, RCD server address or RCD information) for the target IMPU.

Table 5.1-1 summarizes the SBI services produced by the IMS AS and the corresponding APIs defined for this specification.

Table 5.1-1: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |
| Nimsas_SessionEventControl | 6.1 | IMS AS SessionEventControl Service | TS29175_Nimsas_SessionEventControl.yaml | nimsas-sec | A.2 |
| Nimsas_MediaControl | 6.2 | IMS AS MediaControl Service | TS29175_Nimsas_MediaControl.yaml | nimsas-mc | A.3 |
| Nimsas_ImsSessionManagement | 6.3 | IMS AS ImsSessionManagement Service | TS29175_Nimsas_ImsSessionManagement.yaml | nimsas-ism | A.4 |
| Nimsas_EventExposure | 6.4 | IMS AS EventExposure Service | TS29.175_Nimsas_EventExposure.yaml | nimsas-ee | A.5 |
| Nimsas_ImsParameterProvision | 6.5 | IMS AS ImsParameterProvision Service | TS29.175_Nimsas_ImsPP.yaml | nimsas-ipp | A.x |

5.2	Nimsas_SessionEventControl Service

5.2.1	Service Description

The Nimsas_SessionEventControl service enables the consumer to be notified about session events when served IMS subscribers take part in IMS sessions.

5.2.2	Service Operations

5.2.2.1	Introduction

The Service operations defined for the Nimsas_SessionEventControl service are as follows:

-	Notify: It allows the IMS AS to notify the IMS session control events of a specific IMS subscriber to NFs. This service operation reports the event to the consumer that has subscribed implicitly.

NOTE:	Explicit subscription to receive session events is another service operation defined in TS 23.228 [14] which has not specified in this Release. In this Release an implicit subscription is assumed where the IMS AS notifies a configured or discovered the NF consumer of a call event through a Notify.

Table 5.2.2.1-1: Service operations supported by the Nimsas_SessionEventControl service

| Service Operations | Description | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Notify | Notification about IMS session control events of a specific IMS subscriber. | Subscribe/Notify | DCSF |

5.2.2.1A	Subscribe

This is a pseudo operation, the IMS AS does not actually provide Subscribe service operation through Nimsas_SessionEventControl service in this Release. The actual subscription is implicitly subscribed with the SessionEventNotificationUri.

5.2.2.2	Notify

5.2.2.2.1	General

This service operation is invoked by IMS AS and used to enables the IMS AS to notify the NF service consumers (e.g. DCSF) of IMS session events related to a specific served IMS subscriber requesting use of IMS media (e.g. data channel media).

5.2.2.2.2	Notification for Session Event

Figure 5.2.2.2.2-1: Notification for Session Event

1.	If the IMS AS observes the IMS session control events of a specific IMS subscriber that requesting use of IMS data channel media, and it determines based on the IMS subscriber's service profile that the events need to be notified to the DCSF, the IMS AS shall send a POST request to the SessionEventNotificationUri as specified in clause 6.1.5.2.2.

The IMS AS discovers the DCSF and its session event notification URI based on local configuration or via NRF, as specified in clause AC.7.1 of 3GPP TS 23.228 [14].

Based on the operators's policy, if the terminating IMS AS observes the IMS session control events based on the terminating IMS subscriber's DC subscription and DC capability that the events need to be notified to the DCSF, the IMS AS shall send a POST request to the SessionEventNotificationUri as specified in clause 6.1.5.2.2.

2a.	Upon success, the NF Service Consumer responds with "204 No Content". The NF service consumer may invoke the Nimsas_MediaControl service before sending the session event notification response to IMS AS if the NF service consumer determines that media resources for data channel are needed.

2b.	On failure or redirection:

-	If the NF Service Consumer does not consider the "SessionEventNotificationUri" as a valid notification URI, the NF Service Consumer shall return "404 Not Found" status code with the ProblemDetails IE providing details of the error.

-	In the case of redirection, the NF service consumer shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another NF service consumer endpoint. A RedirectResponse IE may be included in the content of POST response.

5.3	Nimsas_MediaControl Service

5.3.1	Service Description

The Nimsas_MediaControl service enables the consumer to control IMS AS handling of media flow within an IMS session. The service can be used by the consumer before responding to a Nimsas_SessionEventControl_Notity request.

If interworking between DC capable UE and non-DC capable UE is supported, the Nimsas_MediaControl shall support to indicate transforming data channel media to video media.

5.3.2	Service Operations

5.3.2.1	Introduction

The Service operations defined for the Nimsas_MediaControl service are as follows:

-	MediaInstruction: It provides instructions to the IMS AS for all media flows a consumer wants to control based on its policies for the received IMS session event, and that may require media resource reservation on MF.

NOTE:	For the case when a specific media flow needs to be terminated in MF media resource (i.e. termination of a Data Channel media descriptor offered by one of the IMS subscribers) or originated by the MF (i.e. origination of a data channel media descriptor to be offered towards one of the IMS subscribers), the consumer must provide a complete MF media specification including information required by MF to know how to terminate or originate the media flow.

Table 5.3.2.1-1: Service operations supported by the Nimsas_MediaControl service

| Service Operations | Description | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| MediaInstruction | Instructions to the IMS AS for all media flows a consumer wants to control based on its policies for the received IMS session event | Request/Response | DCSF |

5.3.2.2	MediaInstruction

5.3.2.2.1	General

This service operation provides instructions to the IMS AS for all media flows a consumer wants to control based on its policies for the received IMS session event, and that may require resource reservation in media resource

The MediaInstruction service operation defined for Nimsas_MediaControl service is used:

-	to enable the NF consumer to instruct the IMS AS how to manage the MF media resource. E.g., setting up the bootstrap data channel with MF both for originating and terminating side, terminating the media flow of the originating UE to MF and updating the MF resource with MDC2 media endpoint information of DC Application Server.

5.3.2.2.2	Media Instruction

Figure 5.3.2.2.2-1 shows a scenario where the NF service consumer (e.g. DCSF) sends a request to the IMS AS to instruct the IMS AS how to manage the MF media resource. The request contains the IMS session identity (/{sessionId}) which is assigned by the IMS AS during the IMS session setup and will be notified to consumer via Nimsas_SessionEventControl notify service operation. After that, the consumer shall reuse the session ID it received from the IMS AS for referencing the same session.

Figure 5.3.2.2.2-1: Media instruction from NF service consumer

1.	The NF service consumer shall send a POST request to the resource representing the individual session resource in the IMS AS. The payload body of the POST request shall contain the media instructions for all the media flows that the NF Service Consumer want to control based on its policies for the received IMS session event via Nimsas_SessionEventControl notification.

For the scenario of interworking via MF, the NF service consumer should instruct the data channel media with "TERMINATE_MEDIA" in data type MediaInstructions and indicate to negotiate the video media with "audioVideoReNegotiationInd" in data type MediaInstructions.

2a.	Upon success, the IMS AS responds with "200 OK" with the message body containing a representation of the created media instruction data.

2b.	On failure or redirection:

In the case of failure, one of the HTTP status code listed in Table 6.2.3.2.4.2.2-2 shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in Table 6.2.3.2.4.2.2-2.

In the case of redirection, the NF service producer shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another NF service producer endpoint.

5.4	Nimsas_ImsSessionManagement Service

5.4.1	Service Description

The Nimsas_ImsSessionManagement service enables the NF service consumer to request IMS AS to create or release an IMS session or modify the media in a specific IMS session, or enables the IMS AS to notify the IMS session events to the interested NF service consumer.

5.4.2	Service Operations

5.4.2.1	Introduction

The service operations defined for the Nimsas_ImsSessionManagement service are as follows:

-	Create: It allows the NF consumer to create an IMS session with standalone data channel media.

-	Update: It allows the the NF consumer to modify the media in a specific IMS session, such as adding, updating or removing the corresponding data channel media.

-	Delete: It allows the NF consumer to release the specific IMS session with standalone data channel media.

-	Notify: It allows the IMS AS to notify the subscribed IMS session events to the NF consumer.

Table 5.4.2.1-1: Service operations supported by the Nimsas_ImsSessionManagement service

| Service Operations | Description | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Create | Create an IMS session with standalone data channel media. | Request/Response | NEF, trusted AF |
| Update | Modify the media in a specific IMS session. | Request/Response | NEF, trusted AF |
| Delete | Release a specific IMS session with standalone data channel media. | Request/Response | NEF, trusted AF |
| Notify | Notify the IMS session events. | Subscribe/Notify | NEF, trusted AF |

5.4.2.2	Create Service Operation

5.4.2.2.1	General

This service operation is used by the service consumer (e.g AF, NEF) to create an IMS session with standalone data channel media.

5.4.2.2.2	Creation of a new IMS session

Figure 5.4.2.2.2-1: Creation of a new IMS session

1.	The NF Service Consumer shall send a POST request to create an "Individual IMS Session" resource in the IMS AS. The payload body of the POST request shall contain a representation of the individual IMS session resource (ImsSessionInfo) to be created.

If the "notifUri" and "notifCorrelationId" attributes are included within the "ImsSessionInfo" data type, this request creates an implicit subscription to IMS session event notification related to this session.

2a.	Upon the reception of the HTTP POST request, if the request is accepted and no error occur, the IMS AS shall create a new session ID for the session and the mediaCorrelationId for each media used in the IMS session.

The IMS AS shall include a HTTP Location header to provide the location of a newly created resource (ImsSessionInfo) together with the status code 201 Created indicating the requested resource is created in the response message.

2b.	On failure, one of the HTTP status code listed in table 6.3.3.2.3.1-3 shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in table 6.3.3.2.3.1-3.

On redirection, the IMS AS shall include 3xx status code, which shall contain a Location header with an URI pointing to the endpoint to another IMS AS (service) instance.

5.4.2.3	Update Service Operation

5.4.2.3.1	General

This service operation is used by the service consumer (e.g. AF, NEF) to modify the media in a specific IMS session, such as creating or closing application data channel(s).

5.4.2.3.2	Partial updating an existing IMS session

Figure 5.4.2.3.2-1: Update an IMS session

1.	The NF Service Consumer shall send a PATCH request to modify an "Individual IMS Session" resource in the IMS AS. The modification may be for adding, updating or deleting a media in the existing IMS session.

For adding, removing or updating a media in an existing ImsSessionInfo, the payload body of the PATCH request shall contain an "add", "remove" or "replace" PATCH operation respectively, with one item of the attribute "mediaInfoSet" of the ImsSessionInfo. The IMS AS shall use the mediaCorrelationId to check whether the operated media exists in the IMS session.

If the payload of the PATCH request includes the "notifUri" and "notifCorrelationId" within the "ImsSessionInfo" data type, this request creates an implicit subscription to IMS session event notification related to this session.

2a.	On success, if the change is to delete the existing media and IMS AS accepts the request change, the IMS AS shall return the status code 204 No Content.

2b. On success, if the change is to add a new media or to update the existing media within an IMS session and the IMS AS accepts the request change, the IMS AS shall return the status code 200 OK. The response shall contain the new resource representation of the resource Individual IMS Session, which includes the added and modified resource or its sub-resource.

2c	On failure, one of the HTTP status code listed in table 6.3.3.3.3.1-3 indicating the error shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in table 6.3.3.3.3.1-3.

On redirection, the IMS AS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint to another IMS AS (service) instance.

5.4.2.3.3	Updating an existing IMS session

Figure 5.4.2.3.3-1: Updating an existing IMS session

1.	The NF Service Consumer shall send a PUT request with the Resource URI "{apiRoot}/nimsas-ism/<apiVersion>/ims-sessions/{sessionId} " to update an "existing IMS session" resource in the IMS AS identified by the {sessionId}.

2a.	On success, the IMS AS shall return the status code "204 No Content" or "200 OK" with the resource representation of the updated resource Individual Ims Event Subscription.

2b	On failure, one of the HTTP status code listed in table 6.4.3.3.3.2-3 indicating the error shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in table 6.4.3.3.3.2-3.

On redirection, the IMS AS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint to another IMS AS (service) instance.

5.4.2.4	Delete Service Operation

5.4.2.4.1	General

This service operation is used by the service consumer (e.g. AF, NEF) to release the specific IMS session.

5.4.2.4.2	Releasing an exisiting IMS Session

Figure 5.4.2.4.2-1: Releasing an existing IMS Session

1.	The NF service consumer shall send a DELETE request to delete an existing Individual IMS Session resource in the IMS AS.

2a.	On success, the request is accepted, the IMS AS shall reply with the status code "204 No Content" indicating that the resource identified by sessionId is successfully deleted in the response message.

2b.	On failure, one of the HTTP status code listed in table 6.3.3.3.3.2-3 indicating the error shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in table 6.3.3.3.3.2-3.

On redirection, the IMS AS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint to another IMS AS (service) instance.

5.4.2.5	Notify

5.4.2.5.1	General

This service operation is invoked by IMS AS and used to enable the IMS AS to notify the NF service consumers (e.g. NEF, trusted AF) of IMS session events related to a specific IMS subscriber.

5.4.2.5.2	Notification for IMS Session Event

Figure 5.4.2.5.2-1: Notification for IMS Session Event

1.	If the IMS AS detects the IMS session events of a specific IMS subscriber, and it determines based on the event subscriptions that the IMS session events need to be notified to the NF service consumer, the IMS AS shall send a POST request to the notifUri as specified in clause 6.3.5.2.2.

2a.	Upon success, the NF Service Consumer responds with "204 No Content".

2b.	On failure or redirection:

-	If the NF Service Consumer does not consider the "notifUri" as a valid notification URI, the NF Service Consumer shall return "404 Not Found" status code with the ProblemDetails IE providing details of the error.

-	In the case of redirection, the NF service consumer shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another NF service consumer endpoint. A RedirectResponse IE may be included in the content of POST response.

5.5	Nimsas_EventExposure Service

5.5.1	Service Description

The Nimsas_EventExposure service enables the service consumer (e.g. HSS on behalf of NEF) to subscribe to subscriber related events so that originally subscribing NF (e.g. NEF) is notified about the subscribed events detected by IMS AS (see 3GPP TS 23.228 [14]).

The Nimsas_EventExposure service enables the service consumer (e.g. NEF) to subscribe to the non-subscriber related events (e.g. any UE) and to be notified about the subscribed events detected by IMS AS (see 3GPP TS 23.228 [14]).

5.5.2	Service Operations

5.5.2.1	Introduction

The service operations defined for the Nimsas_EventExposure service are as follows:

-	Subscribe: It allows the NF consumer to subscribe for a specific subscriber related IMS event.

-	Unsubscribe: It allows the the NF consumer to delete the subscription for a specific subscriber or non-subscriber related event.

-	Notify: It allows the IMS AS to notify the specific subscriber or non-subscriber related event to NFs (e.g. NEF). This service operation reports the information associated with a subscribed event.

Table 5.5.2.1-1: Service operations supported by the Nimsas_EventExposure service

| Service Operations | Description | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Subscribe | Subscribe to a specific subscriber related IMS event. | Subscribe/Notify | NEF, HSS, trusted AF |
| Unsubscribe | Delete the subscription to a specific subscriber related IMS event. | Subscribe/Notify | NEF, HSS, trusted AF |
| Notify | Notification about events detected by IMS AS. | Subscribe/Notify | NEF, trusted AF |

5.5.2.2	Subscribe

5.5.2.2.1	General

This service operation is used by the service consumer (e.g HSS, NEF) to subscribe the IMS events on the IMS AS or update an existing subscription on the IMS AS.

The IMS AS may offer this service as a Service Producer to enable an NF to subscribe to event notifications, e.g. on behalf of another NF, and get notified about an event.

The IMS AS shall include the events it supports as part of its NF profile registered in NRF.

NOTE:	The events supported by IMS AS in the NF profile can be known or unknown. The NF consumer (e.g. HSS) does not need to know the semantics of an unknown event.

5.5.2.2.2	Subscription for event notifications

Figure 5.5.2.2.2-1: Creation of a new subscription

1.	The NF Service Consumer shall send a POST request to create an "Individual IMS Event Subscription" resource in the IMS AS. The payload body of the POST request shall contain a representation of the individual IMS event subscription resource (ImsEventsSubscription) to be created. The request may contain an expiry time representing the time upto which the subscription is to be kept active (i.e. the time after which the subscribed event(s) shall stop generating reports).

2a.	Upon the reception of the HTTP POST request, if the request is accepted and no error occur, the IMS AS shall create a new subscription ID for the subscription.

The IMS AS shall include a HTTP Location header to provide the location of a newly created resource (ImsEventsSubscriptionCreated) together with the status code "201 Created" indicating the requested resource is created in the response message. The response may include an expiry time, which may be equal to the requested expiry time or lower.

2b.	On failure, one of the HTTP status code listed in table 6.4.3.2.3.1-3 shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in table 6.4.3.2.3.1-3.

On redirection, the IMS AS shall include 3xx status code, which shall contain a Location header with an URI pointing to the endpoint to another IMS AS (service) instance.

5.5.2.2.3	Updating an exisiting subscription

Figure 5.5.2.2.3-1: Updating an existing subscription

1.	The NF Service Consumer shall send a PUT request with the Resource URI "{apiRoot}/nimsas-ee/<apiVersion>/subscriptions/{subscriptionId}" to update an "Individual IMS Event Subscription" resource in the IMS AS identified by the {subscriptionId}. The ImsEventsSubscription data structure provided in the request body shall include the same contents as described in clause 5.5.2.2.2.

2a.	On success, the IMS AS shall return the status code "204 No Content" or "200 OK" with the resource representation of the updated resource Individual Ims Event Subscription.

2c	On failure, one of the HTTP status code listed in table 6.4.3.3.3.2-3 indicating the error shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in table 6.4.3.3.3.2-3.

On redirection, the IMS AS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint to another IMS AS (service) instance.

5.5.2.2.4	Subscription Partial Update

Figure 5.5.2.2.4-1: Partially update a subscription

1.	The NF Service Consumer shall send a PATCH request to modify an "Individual IMS Event Subscription" resource in the IMS AS identified by the {subscriptionId}. The content of the PATCH request shall contain the list of operations (add/delete/replace) to be applied to the subscription; these operations may be directed to individual parameters of the subscription or to the list of the subscribed events.

2a.	On success, if the change is to delete the events to be subscribed and IMS AS accepts the request change, the IMS AS shall return the status code 204 No Content.

2b. On success, if the change is to add new events to be subscribed or to update the existing subscribed event within a subscription and the IMS AS accepts the request change, the IMS AS shall return the status code 200 OK. The response shall contain the new resource representation of the resource Individual IMS Event Subcription.

2c	On failure, one of the HTTP status code listed in table 6.4.3.3.3.3-3 indicating the error shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in table 6.4.3.3.3.3-3.

On redirection, the IMS AS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint to another IMS AS (service) instance.

5.5.2.3	Unsubscribe

5.5.2.3.1	General

This service operation is used by the service consumer (e.g. HSS, NEF) to unsubscribe from event notifications.

5.5.2.3.2	Unsubscribe from notifications

Figure 5.5.2.3.2-1: Unsubscribe from notifications

1.	The NF service consumer shall send a DELETE request to delete an existing Individual IMS Event Subscription resource in the IMS AS.

2a.	On success, the request is accepted, the IMS AS shall reply with the status code "204 No Content" indicating that the resource identified by subscriptionId is successfully deleted in the response message.

2b.	On failure, one of the HTTP status code listed in table 6.4.3.3.3.2-3 indicating the error shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in table 6.4.3.3.3.2-3.

On redirection, the IMS AS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint to another IMS AS (service) instance.

5.5.2.3	Notify

5.5.2.3.1	General

This service operation is used by the IMS AS to enables the IMS AS to notify the NF service consumers (e.g. HSS, NEF) of IMS session events related to a specific served IMS subscriber requesting use of IMS media (e.g. data channel media).

5.5.2.2.2	Notification for Session Event

Figure 5.2.2.2.2-1: Notification for Session Event

1.	If the IMS AS observes the IMS events of a specific IMS subscriber, the IMS AS shall send a POST request to the request with "{notificationURI}" received in the Nims_ImsEE_Subscribe service operation as Resource URI.

2a.	Upon success, the NF Service Consumer responds with "204 No Content".

2b.	On failure or redirection:

-	If the NF Service Consumer does not consider the "notificationUri" as a valid notification URI, the NF Service Consumer shall return "404 Not Found" status code with the ProblemDetails IE providing details of the error.

-	In the case of redirection, the NF service consumer shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another NF service consumer endpoint. A RedirectResponse IE may be included in the content of POST response.

5.5.2.4	SubscriptionInfoRetrieval

5.5.2.4.1	General

This service operation allows the retrieval of subscription informaiton.

5.5.2.4.2	Subscription Info Retrieval

Figure 5.5.2.4.2-1: Subscription info retrieval

1.	The NF Service Consumer shall send an HTTP GET request to the resource URI "subscription" collection resource.

2a.	On success, the IMS AS shall return the status code 200 OK. The response shall contain the resource representation of the resource Individual IMS Event Subcription.

2b.	On failure, one of the HTTP status code listed in table 6.4.3.3.3.1-3 indicating the error shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in table 6.4.3.3.3.1-3.

On redirection, the IMS AS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint to another IMS AS (service) instance.

5.6	Nimsas_ImsParameterProvision Service

5.6.1	Service Description

The Nimsas_ImsParameterProvision service enables the service consumer (e.g. the NEF or trusted AF) to provision the IMS user related information for the target IMPU to support the 3rd party user identity information (see 3GPP TS 23.228 [14]).

5.6.2	Service Operations

5.6.2.1	Introduction

The service operations defined for the Nimsas_ImsParameterProvision service are as follows:

-	Create: It allows the NF consumer to provision the IMS user related information for a target subscriber.

-	Update: It allows the NF consumer to update the IMS user related information for a target subscriber.

-	Delete: It allows the NF consumer to delete the IMS user related information for a target subscriber.

Table 5.6.2.1-1: Service operations supported by the Nimsas_ImsParameterProvision service

| Service Operations | Description | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Create | Create the IMS user related information for a target subscriber. | Request/Response | NEF, trusted AF |
| Update | Update the IMS user related information for a target subscriber. | Request/Response | NEF, trusted AF |
| Delete | Delete the IMS user related information for a target subscriber. | Request/Response | NEF, trusted AF |

5.6.2.2	Create Service Operation

5.6.2.2.1	General

This service operation is used by the service consumer (e.g. the NEF) to create the IMS Parameter Provisioning Data entry for a certain AF which contains the IMS user related information (e.g., RCD URL, RCD server address or RCD information for the target IMPU).

5.6.2.2.2	Creation of the Parameter Provisioning Data Entry per AF

The request contains the IMS UE's identity (/{imsUeId}) which shall be an IMPU.

Figure 5.6.2.2.2-1: Creation of the IMS Parameter Provisioning Data Entry per AF

1.	The NF Service Consumer (e.g. the NEF) sends a PUT request to the resource that represents the new IMS Parameter Provisioning Data entry for the AF identified by the afInstanceId. The request body shall contain an ImsPpDataEntry object representing the value of the new resource.

2a.	Upon the reception of the HTTP PUT request, if the request is accepted and no error occurs, the IMS AS shall respond with "201 Created" and the response body shall contain the ImsPpDataEntry object representing the new created resource.

2b.	On failure, one of the HTTP status codes listed in table 6.5.3.2.3.1-3 shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in table 6.5.3.2.3.1-3.

5.6.2.3	Update Service Operation

5.6.2.3.1	General

This service operation is used by the service consumer (e.g. the NEF) to modify the IMS Parameter Provisioning Data entry for a certain AF which contains the IMS user related information (e.g., RCD URL, RCD server address or RCD information for the target IMPU).

5.6.2.3.2	Updating the Parameter Provisioning Data Entry per AF

The request contains the IMS UE's identity (/{imsUeId}) which shall be an IMPU.

Figure 5.6.2.3.2-1: Update the IMS Parameter Provisioning Data Entry per AF

1.	The NF Service Consumer shall send a PUT request to modify the resource that represents the IMS Parameter Provisioning Data entry for the AF identified by the afInstanceId. The request body shall contain an ImsPpDataEntry object representing the value of the modified resource.

2a., 2b.	On success, the IMS AS shall return the status code "200 OK" with the resource representation of the updated resource ImsPpDataEntry or "204 No Content".

2c.	On failure, one of the HTTP status codes listed in table 6.5.3.3.3.1-3 indicating the error shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in table 6.5.3.3.3.1-3.

5.6.2.4	Delete Service Operation

5.6.2.4.1	General

This service operation is used by the service consumer (e.g. AF, NEF) to delete the IMS user related information for a target subscriber.

5.6.2.4.2	Deleting the Parameter Provisioning Data Entry per AF

Figure 5.6.2.4.2-1: Deleting the IMS Parameter Provisioning Data Entry per AF

1.	The NF service consumer shall send a DELETE request to delete the resource that represents the IMS Parameter Provisioning Data entry for the AF identified by the afInstanceId.

2a.	On success, the request is accepted, the IMS AS shall reply with the status code "204 No Content" indicating that the resource identified by ImsPpDataEntry is successfully deleted in the response message.

2b.	On failure, one of the HTTP status codes listed in table 6.5.3.3.3.2-3 indicating the error shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in table 6.5.3.3.3.2-3.

6	API Definitions

6.1	Nimsas_SessionEventControl Service API

6.1.1	API URI

The Nimsas_SessionEventControl service shall use the Nimsas_SessionEventControl API.

The API URI of the Nimsas_SessionEventControl API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests from the NF service consumer towards the NF service producer shall have the Resource URI structure defined in clause 4.4.1 of 3GPP TS 29.501 [5], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [5].

-	The <apiName> shall be "nimsas-sec".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 6.1.3.

6.1.2	Usage of HTTP

6.1.2.1	General

HTTP/2, IETF RFC 7540 [11], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

The OpenAPI [6] specification of HTTP messages and content bodies for the Nimsas_SessionEventControl API is contained in Annex A.

6.1.2.2	HTTP standard headers

6.1.2.2.1	General

See clause 5.2.2 of 3GPP TS 29.500 [4] for the usage of HTTP standard headers.

6.1.2.2.2	Content type

JSON, IETF RFC 8259 [12], shall be used as content type of the HTTP bodies specified in the present specification as specified in clause 5.4 of 3GPP TS 29.500 [4]. The use of the JSON format shall be signalled by the content type "application/json".

"Problem Details" JSON object shall be used to indicate additional details of the error in a HTTP response body and shall be signalled by the content type "application/problem+json", as defined in IETF RFC 7807 [13].

6.1.2.3	HTTP custom headers

The mandatory HTTP custom header fields specified in clause 5.2.3.2 of 3GPP TS 29.500 [4] shall be supported, and the optional HTTP custom header fields specified in clause 5.2.3.3 of 3GPP TS 29.500 [4] may be supported.

6.1.3	Resources

6.1.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.1.3.1-1 depicts the resource URIs structure for the Nimsas_SessionEventControl service API.

Figure 6.1.3.1-1: Resource URI structure of the Nimsas_SessionEventControl API

Table 6.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.1.3.1-1: Resources and methods overview

| Resource purpose/name | Resource URI (relative path after API URI) | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Session Event Subscriptions (Collection) | /session-event-subscriptions | POST | This is a pseudo resource. |

6.1.3.2	Resource: Session Event Subscriptions

6.1.3.2.1	Description

6.1.3.2.2	Resource Definition

Resource URI: {apiRoot}/nimsas-sec/<apiVersion>/session-event-subscriptions

This resource shall support the resource URI variables defined in table 6.1.3.2.2-1.

Table 6.1.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1 |

6.1.3.2.3	Resource Standard Methods

6.1.3.2.3.1	POST

This method shall support the URI query parameters specified in table 6.1.3.2.3.1-1.

Table 6.1.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.3.2.3.1-3.

Table 6.1.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| Any |  |  |  |

Table 6.1.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |
| NOTE: The mandatory HTTP error status code for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

6.1.3.2.4	Resource Custom Operations

None.

6.1.4	Custom Operations without associated resources

None in this release of the specification.

6.1.5	Notifications

6.1.5.1	General

Notifications shall comply to clause 6.2 of 3GPP TS 29.500 [4] and clause 4.6.2.3 of 3GPP TS 29.501 [5].

Table 6.1.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Session Event Notification | {SessionEventNotificationUri} | POST | Report the session event observed. |

6.1.5.2	Session Event Notification

6.1.5.2.1	Description

The Session Event Notification is used by the IMS AS to report the observed Session Event to notification endpoints.

6.1.5.2.2	Target URI

The Callback URI "{SessionEventNotificationUri}" shall be used with the callback URI variables defined in table 6.1.5.2.2-1.

Table 6.1.5.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| SessionEventNotificationUri | The notification URI of the Service Consumer (i.e., DCSF) to receive the session events. In this release, the SessionEventNotificationUri of the Service Consumer is locally configured in the IMS AS or discovered by the IMS AS via NRF (in the default notification subscription with the "DC_SESSION_EVENT_NOTIFICATION" notification type). |

6.1.5.2.3	Standard Methods

6.1.5.2.3.1	POST

This method shall support the request data structures specified in table 6.1.5.2.3.1-1 and the response data structures and response codes specified in table 6.1.5.2.3.1-1.

Table 6.1.5.2.3.1-1: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SessionEventNotification | M | 1 | The session event notification to notification endpoints. |

Table 6.1.5.2.3.1-2: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | This case represents a successful notification of the event. |
| ProblemDetails | O | 0..1 | 404 Not Found | Indicates the session event notification has failed due to application error. The "cause" attribute may be used to indicate one of the following application errors: - USER_NOT_FOUND, e.g. if the NF Service Consumer (e.g. DCSF) does not serve this service user; - NOTIFICATION_URI_NOT_FOUND, if the NF Service Consumer (e.g. DCSF) considers the "SessionEventNotificationUri" is not recognized. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.1.5.2.3-3: Headers supported by the 307 Response Code on this endpoint

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | A URI pointing to the endpoint of the NF service consumer instance to which the request should be sent. For the case, when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF instance ID towards which the notification is redirected |

Table 6.1.5.2.3-4: Headers supported by the 308 Response Code on this endpoint

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | A URI pointing to the endpoint of the NF service consumer instance to which the request should be sent. For the case, when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF instance ID towards which the notification is redirected |

6.1.6	Data Model

6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nimsas_SessionEventControl service based interface protocol.

Table 6.1.6.1-1: Nimsas_SessionEventControl specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| SessionEventNotification | 6.1.6.2.2 | The session event notification item. |
| NotificationEvent | 6.1.6.2.3 | The notification event information. |
| SessionInfo | 6.1.6.2.4 | The session information related to the notification event. |
| MediaInfo | 6.1.6.2.5 | The media information related to the notification event. |
| DcMediaSpec | 6.1.6.2.6 | The data channel media specification information which includes media description attributes. |
| QosHintInfo | 6.1.6.2.7 | The QoS requirement of the media. |
| EventType | 6.1.6.3.3 | The type of the event for which the notification is generated. |
| MediaType | 6.1.6.3.4 | The type of the media for which the notification is generated. |
| SessionCase | 6.1.6.3.5 | The type of the session case. |
| EventInitiator | 6.1.6.3.6 | The initiator of the event for which the notification is generated. |
| PsDataOffStatus | 6.1.6.3.7 | The active/inactive status of PS Data Off for a served UE. |

Table 6.1.6.1-2 specifies data types re-used by the Nimsas_SessionEventControl service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nimsas_SessionEventControl service based interface.

Table 6.1.6.1-2: Nimsas_SessionEventControl re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| ImsPublicId | 3GPP TS 29.562 [15] | IMS Public Identity. |
| DcStream | 3GPP TS 29.571 [16] | Data Channel mapping and configuration information. |
| MaxMessageSize | 3GPP TS 29.571 [16] | Maximum message size of to be expected. |
| DcEndpoint | 3GPP TS 29.571 [16] | local or remote endpoint for the Data Channel. |
| SessionId | 3GPP TS 29.571 [16] | IMS Session Identity. |
| MediaId | 3GPP TS 29.571 [16] | IMS media flow identity. |
| BdcUsedBy | 3GPP TS 29.571 [16] | The party using the bootstrap data channel. |
| AppBindingInfo | 3GPP TS 29.571 [16] | The application binding information of the application data channel. |
| MediaInfoExternal | 6.3.6.2.4 | The media related information. |

6.1.6.2	Structured data types

6.1.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

6.1.6.2.2	Type: SessionEventNotification

Table 6.1.6.2.2-1: Definition of type SessionEventNotification

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| notificationEvent | NotificationEvent | M | 1 | The NotificationEvent corresponds to a report on one subscribed event. |
| sessionId | SessionId | M | 1 | The session ID is the identity of the IMS session for which the event relates to. This IE contains the information in the Call-ID header which is the typical header of SIP message. |
| sessionInfo | SessionInfo | C | 0..1 | The sessionInfo is the IMS session related information, e.g. the calling and called identity. It shall be contained when the attribute eventType of the notificationEvent is one of the "SESSION_ESTABLISHMENT_REQUEST", "MEDIA_CHANGE_REQUEST", "EXTERNAL_SESSION_CREATE", "SESSION_ESTABLISHMENT_SUCCESS" (NOTE). |
| mediaInfoList | map(MediaInfo) | C | 1..N | Media info list includes a list of media related information, e.g. media identity, media type and media specifications. The mediaId attribute within the MediaInfo data type is the key of the map. It shall be contained when the attribute eventType of the notificationEvent is one of the "SESSION_ESTABLISHMENT_REQUEST", "SESSION_ESTABLISHMENT_PROGRESS", "SESSION_ESTABLISHMENT_ALERTING", "SESSION_ESTABLISHMENT_SUCCESS", "MEDIA_CHANGE_REQUEST", "MEDIA_CHANGE_SUCCESS", "MEDIA_CHANGE_FAILURE", or "MEDIA_CHANGE_CANCEL". |
| psDataOff | PsDataOffStatus | C | 0..1 | It indicates the active/inactive status of PS Data Off for the served UE. It shall be present if the attribute eventType of the notificationEvent is set to one of the following: - "SESSION_ESTABLISHMENT_REQUEST", when IMS session establishment is invoked; - "PS_DATA_OFF_STATUS_CHANGE", when the status change of PS Data Off happens without any other changes to the IMS session; It may be present in other cases if the change of PS Data Off status happens together with other events (e.g. media changes). |
| mediaInfoSet | array(MediaInfoExternal) | C | 0..1 | Media information set includes a set of media information. It shall be contained when the attribute eventType of the notificationEvent is one of the "EXTERNAL_SESSION_UPDATE" "EXTERNAL_SESSION_CREATE" |
| NOTE: The sessionInfo shall be present when the attribute eventType of the notificationEvent is set to "SESSION_ESTABLISHMENT_SUCCESS" and the originating IMS AS received the redirected-to party identity via the History-Info header field in the 200 OK responses to the initial INVITE request. |  |  |  |  |

6.1.6.2.3	Type: NotificationEvent

Table 6.1.6.2.3-1: Definition of type NotificationEvent

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| eventType | EventType | M | 1 | The event type of the event for which the notification is generated. |
| eventInitiator | EventInitiator | C | 0..1 | Event initiator indicates initiator of the event, e.g. "served IMS subscriber", "remote IMS subscriber" or "external party". It shall be contained when the eventType is set to one of the following: - "SESSION_ESTABLISHMENT_REQUEST", - "MEDIA_CHANGE_REQUEST", - "EXTERNAL_SESSION_CREATE". |

6.1.6.2.4	Type: SessionInfo

Table 6.1.6.2.4-1: Definition of type SessionInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| callingIdentity | ImsPublicId | O | 0..1 | The IMS public identity of the calling IMS subscriber, or a specific Public Service Identity. (NOTE 1) |
| calledIdentity | ImsPublicId | O | 0..1 | The IMS public identity of the called IMS Subscriber, or a specific Public Service Identity of DCSF. (NOTE 2) |
| sessionCase | SessionCase | O | 0..1 | Indicator to indicate that whether this is an originating IMS session, a terminating IMS session or external initiated IMS session. |
| NOTE 1: The value of the callingIdentity shall be set to the value of the P-Asserted-Identity header field of the received initial INVITE request, see 3GPP TS 24.186 [19] clause 9.3.2.2 and clause 9.3.3.2. NOTE 2: The value of the calledIdentity shall be set to the Request URI of the received initial INVITE request, see 3GPP TS 24.186 [19] clause 9.3.2.2 and clause 9.3.3.2. If communication diversion occurs and the originating IMS AS received the History-Info header field in after receiving the 200 OK responses to the initial INVITE request, the originating IMS AS shall set the calledIdentity attribute based on the received History-Info header field, see 3GPP TS 24.604 [21] clause 4.5.2.7. |  |  |  |  |

6.1.6.2.5	Type: MediaInfo

Table 6.1.6.2.5-1: Definition of type MediaInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| mediaId | MediaId | M | 1 | The media ID uniquely identifies this media item within the list. The identity is allocated by IMS AS. |
| mediaType | MediaType | M | 1 | The media type of the media for which the notification is generated. |
| dcMediaSpec | DcMediaSpec | C | 0..1 | The data channel media specification includes the relevant media attributes of interest to the consumer. It shall be contained if the mediaType is set to "DC". |
| mediaSuspended | boolean | C | 0..1 | This IE indicates whether the media is suspended or not. It shall be contained if the media is suspended or resumed as specified in 3GPP TS 24.610 [18] and 3GPP TS 24.186 [19]. - true: the media is suspended. - false: the media is resumed. |
| mediaRemovalInd | boolean | C | 0..1 | This IE indicates whether the media is to be removed or not. It shall be only present with true value if the media is to be removed, e.g. removal of bootstrap data channel media during the interaction with supplementary service, or removal of application data channel in other cases. Presence with false value shall be prohibited. |

6.1.6.2.6	Type: DcMediaSpec

Table 6.1.6.2.6-1: Definition of type DcMediaSpec

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| streams | map(DcStream) | M | 1..N | Represents the data channel mapping and configuration information when originating/terminating data channel media flows on the Mb interface. The streamId attribute within the DcStream data type is the key of the map. (NOTE) |
| maxMessageSize | MaxMessageSize | O | 0..1 | Represents the maximum message size of to be expected. (NOTE) |
| receivedDcEndpoint | DcEndpoint | M | 0..1 | Represents the SCTP and DTLS endpoint information for the Data Channel received from the SDP offer/answer. (NOTE) |
| bdcUsedby | BdcUsedBy | C | 0..1 | Represents the party using the bootstrap data channel. It shall be contained when the mediaId in the data type MediaInfo represents bootstrap data channel and the streamId set to 100 and 110. |
| appBindingInfo | AppBindingInfo | C | 0..1 | Represents the application binding information of the application data channel. It shall be contained when the mediaId in the data type MediaInfo represents application data channel. |
| qosHintInfo | QosHintInfo | C | 0..1 | Represents the QoS information that is provided within the SDP media level attribute "a=3gpp-qos-hint". It shall be included if the received SDP offer and SDP answer contains the "a=3gpp-qos-hint" attribute line. |
| NOTE: The attribute and the value are derived from the SDP received by the IMS AS in a SIP INVITE or a re-INVITE request and the corresponding response related to an IMS Data Channel. |  |  |  |  |

6.1.6.2.7	Type: QosHintInfo

Table 6.1.6.2.7-1: Definition of type QosHintInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| loss | number | O | 0..1 | Represents the maximum desirable end-to-end transport level packet loss rate in percent (without "%" sign) as a zero-based-integer or as a non-zero-real value. |
| localLoss | number | O | 0..1 | Represents the maximum desirable end-to-end transport level packet loss rate applied across the local link. It may be included if the loss is included. |
| latency | number | O | 0..1 | Represents the maximum desirable end-to-end transport level packet latency in milliseconds as a zero-based-integer or as a non-zero-real value. |
| localLatency | number | O | 0..1 | Represents the maximum desirable end-to-end transport level packet latency applied across the local link. It may be included if the loss is included. |

6.1.6.3	Simple data types and enumerations

6.1.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

6.1.6.3.2	Simple data types

The simple data types defined in table 6.1.6.3.2-1 shall be supported.

Table 6.1.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

6.1.6.3.3	Enumeration: EventType

The enumeration EventType represents the type of the event for which the notification is generated. It shall comply with the provisions defined in table 6.1.6.3.3-1.

Table 6.1.6.3.3-1: Enumeration EventType

| Enumeration value | Description |
| --- | --- |
| "SESSION_ESTABLISHMENT_REQUEST" | Session establishment request event. |
| "SESSION_ESTABLISHMENT_PROGRESS" | Session establishment progress event. |
| "SESSION_ESTABLISHMENT_ALERTING" | Session establishment alerting event. |
| "SESSION_ESTABLISHMENT_SUCCESS" | Session establishment success event. |
| "SESSION_ESTABLISHMENT_FAILURE" | Session establishment failure event. |
| "SESSION_ESTABLISHMENT_CANCEL" | Session establishment cancel event. |
| "MEDIA_CHANGE_REQUEST" | Media change request event. |
| "MEDIA_CHANGE_SUCCESS" | Media change success event. |
| "MEDIA_CHANGE_FAILURE" | Media change failure event. |
| "MEDIA_CHANGE_CANCEL" | Media change cancel event. |
| "SESSION_TERMINATION" | Session termination event. |
| "EXTERNAL_SESSION_UPDATE" | External session update event. |
| "EXTERNAL_SESSION_CREATE" | External session create event. |
| "PS_DATA_OFF_STATUS_CHANGE" | PS Data Off status change event. |

6.1.6.3.4	Enumeration: MediaType

The enumeration MediaType represents the type of the media for which the notification is generated. It shall comply with the provisions defined in table 6.1.6.3.4-1.

Table 6.1.6.3.4-1: Enumeration MediaType

| Enumeration value | Description |
| --- | --- |
| "DC" | Data channel media type. |
| "AUDIO" | Audio media type. |
| "VIDEO" | Video media type. |

6.1.6.3.5	Enumeration: SessionCase

The enumeration SessionCase represents the type of the session to for which the notification is generated. It shall comply with the provisions defined in table 6.1.6.3.5-1.

Table 6.1.6.3.5-1: Enumeration SessionCase

| Enumeration value | Description |
| --- | --- |
| "ORIGINATING_IMS_SESSION" | Originating IMS session. |
| "TERMINATING_IMS_SESSION" | Terminating IMS session. |
| "EXTERNAL_INITIATED_IMS_SESSION" | External party initiated IMS session. |

6.1.6.3.6	Enumeration: EventInitiator

The enumeration EventInitiator represents the initiator of the event for which the notification is generated. It shall comply with the provisions defined in table 6.1.6.3.6-1.

Table 6.1.6.3.6-1: Enumeration EventInitiator

| Enumeration value | Description |
| --- | --- |
| "SERVED_IMS_SUBSCRIBER" | Served IMS subscriber. |
| "REMOTE_IMS_SUBSCRIBER" | Remote IMS subscriber. |
| "EXTERNAL_PARTY" | External party, i.e. 3rd party. |

6.1.6.3.7	Enumeration: PsDataOffStatus

The enumeration PsDataOffStatus represents the active/inactive status of PS Data Off for a served UE. It shall comply with the provisions defined in table 6.1.6.3.7-1.

Table 6.1.6.3.7-1: Enumeration PsDataOffStatus

| Enumeration value | Description |
| --- | --- |
| "ACTIVE" | PS Data Off status is set to active. |
| "INACTIVE" | PS Data Off status is set to inactive. |

6.1.6.4	Data types describing alternative data types or combinations of data types

None in this release of the specification.

6.1.6.5	Binary data

None in this release of the specification.

6.1.7	Error Handling

6.1.7.1	General

For the Nimsas_SessionEventControl API, HTTP error responses shall be supported as specified in clause 4.8 of 3GPP TS 29.501 [5]. Protocol errors and application errors specified in table 5.2.7.2-1 of 3GPP TS 29.500 [4] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.2.7.1-1 of 3GPP TS 29.500 [4].

In addition, the requirements in the following clauses are applicable for the Nimsas_SessionEventControl API.

6.1.7.2	Protocol Errors

No specific procedures for the Nimsas_SessionEventControl service are specified.

6.1.7.3	Application Errors

The application errors defined for the Nimsas_SessionEventControl service are listed in Table 6.1.7.3-1.

Table 6.1.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| USER_NOT_FOUND | 404 Not Found | The service user is not served by this DCSF, e.g. no data channel configurations for this service user is found. |
| NOTIFICATION_URI_NOT_FOUND | 404 Not Found | The session event notification URI is not recognized by the NF service consumer (i.e. DCSF). |

6.1.8	Feature negotiation

The optional features in table 6.1.8-1 are defined for the Nimsas_SessionEventControl API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 6.1.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

6.1.9	Security

As indicated in 3GPP TS 33.501 [8] and 3GPP TS 29.500 [4], the access to the Nimsas_SessionEventControl API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [9]), based on local configuration, using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [10]) plays the role of the authorization server.

If OAuth2 is used, an NF Service Consumer, prior to consuming services offered by the Nimsas_SessionEventControl API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [10], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Nimsas_SessionEventControl service.

The Nimsas_SessionEventControl API defines a single scope "nimsas-sessioneventcontrol" for the entire service, and it does not define any additional scopes at resource or operation level.

6.1.10	HTTP redirection

An HTTP request may be redirected to a different IMS AS service instance when using direct or indirect communications (see 3GPP TS 29.500 [4]).

An SCP that reselects a different NF consumer (e.g. DCSF) instance will return the NF Instance ID of the new NF consumer instance in the 3gpp-Sbi-Producer-Id header, as specified in clause 6.10.3.4 of 3GPP TS 29.500 [4].

If an NF consumer (e.g. DCSF) redirects a service request to a different NF consumer using an 307 Temporary Redirect or 308 Permanent Redirect status code, the identity of the new NF consumer towards which the service request is redirected shall be indicated in the 3gpp-Sbi-Target-Nf-Id header of the 307 Temporary Redirect or 308 Permanent Redirect response as specified in clause 6.10.9.1 of 3GPP TS 29.500 [4].

6.2	Nimsas_MediaControl Service API

6.2.1	API URI

The Nimsas_MediaControl service shall use the Nimsas_MediaControl API.

The API URI of the Nimsas_MediaControl API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests from the NF service consumer towards the NF service producer shall have the Resource URI structure defined in clause 4.4.1 of 3GPP TS 29.501 [5], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [5].

-	The <apiName> shall be "nimsas-mc".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 6.2.3.

6.2.2	Usage of HTTP

6.2.2.1	General

HTTP/2, IETF RFC 7540 [11], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

The OpenAPI [6] specification of HTTP messages and content bodies for the Nimsas_MediaControl API is contained in Annex A.

6.2.2.2	HTTP standard headers

6.2.2.2.1	General

See clause 5.2.2 of 3GPP TS 29.500 [4] for the usage of HTTP standard headers.

6.2.2.2.2	Content type

JSON, IETF RFC 8259 [12], shall be used as content type of the HTTP bodies specified in the present specification as specified in clause 5.4 of 3GPP TS 29.500 [4]. The use of the JSON format shall be signalled by the content type "application/json".

"Problem Details" JSON object shall be used to indicate additional details of the error in a HTTP response body and shall be signalled by the content type "application/problem+json", as defined in IETF RFC 7807 [13].

6.2.2.3	HTTP custom headers

The mandatory HTTP custom header fields specified in clause 5.2.3.2 of 3GPP TS 29.500 [4] shall be supported, and the optional HTTP custom header fields specified in clause 5.2.3.3 of 3GPP TS 29.500 [4] may be supported.

6.2.3	Resources

6.2.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.2.3.1-1 depicts the resource URIs structure for the Nimsas_MediaControl service API.

Figure 6.2.3.1-1: Resource URI structure of the Nimsas_MediaControl API

Table 6.2.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.2.3.1-1: Resources and methods overview

| Resource purpose/name | Resource URI (relative path after API URI) | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Individual call session (Document) | /call-sessions/{sessionId}/media-instruction | media-instruction (POST) | Instructs the IMS AS on how to control the media resources. |

6.2.3.2	Resource: Individual call session

6.2.3.2.1	Description

This resource represents an individual call session created in the IMS AS.

This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [5]).

6.2.3.2.2	Resource Definition

Resource URI: {apiRoot}/nimsas-mc/<apiVersion>/call-sessions/{sessionId}

This resource shall support the resource URI variables defined in table 6.2.3.2.2-1.

Table 6.2.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.2.1 |
| sessionId | SessionId | Session ID assigned by the IMS AS during the IMS session setup and will be notified to consumer via Nimsas_SessionEventControl service. The consumer shall reuse the session ID it received from the IMS AS for referencing the same session. |

6.2.3.2.3	Resource Standard Methods

None.

6.2.3.2.4	Resource Custom Operations

6.2.3.2.4.1	Overview

Table 6.2.3.2.4.1-1: Custom operations

| Operation Name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| media-instruction | {resourceUri}/media-instruction | POST | Update call session service operation. |

6.2.3.2.4.2	Operation: media-instruction

6.2.3.2.4.2.1	Description

6.2.3.2.4.2.2	Operation Definition

This custom operation updates an individual call session resource and provide a set of media instruction information for a given IMS session, towards IMS AS.

This operation shall support the request data structures specified in table 6.2.3.2.4.2.2-1 and the response data structure and response codes specified in table 6.2.3.2.4.2.2-2.

Table 6.2.3.2.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MediaInstructionData | M | 1 | Representation of the updates to apply to the call session. |

Table 6.2.3.2.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MediaInstructionData | C | 0..1 | 200 OK | Successful update of the call session, when the IMS AS needs to return information in the response. |
| n/a |  |  | 204 No Content | Successful update of the call session, when the IMS AS does not need to return information in the response. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| ProblemDetails | O | 0..1 | 400 Bad Request | Indicates the media instruction has failed due to application error. The "cause" attribute may be used to indicate one of the following application errors: - MEDIA_ID_NOT_FOUND |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.2.3.3.4.2.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | A URI pointing to the endpoint of the NF service consumer instance to which the request should be sent. For the case, when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF instance ID towards which the request is redirected. |

Table 6.2.3.3.4.2.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | A URI pointing to the endpoint of the NF service consumer instance to which the request should be sent. For the case, when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target SMF (service) instance ID towards which the request is redirected. |

6.2.4	Custom Operations without associated resources

None in this release of the specification.

6.2.5	Notifications

None in this release of the specification.

6.2.6	Data Model

6.2.6.1	General

This clause specifies the application data model supported by the API.

Table 6.2.6.1-1 specifies the data types defined for the Nimsas_MediaControl service based interface protocol.

Table 6.2.6.1-1: Nimsas_MediaControl specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| ArMediaSpecification | 6.2.6.2.5 | The AR media specification information needed for AR communication services from application layer. |
| AudioVideoReNegotiationInd | 6.2.6.2.7 | The audio/video media re-negotiation indication information to connect the UE's audio/video media stream to MF. |
| DcMediaSpecification | 6.2.6.2.4 | The data channel media specification information needed for data channel media stream from application layer. |
| InterworkingInstruction | 6.2.6.2.8 | The interworking instruction of data channel transcoding. |
| Mdc2EndpointInfo | 6.2.6.2.6 | The MDC2 media endpoint information. |
| MediaInstruction | 6.2.6.3.3 | The media instruction for handling a specific media flow. |
| MediaInstructionData | 6.2.6.2.2 | The media instruction data for a specific session. |
| MediaInstructions | 6.2.6.2.3 | The media instructions for a specific media flow. |
| MediaConnSide | 6.2.6.3.4 | The side that the audio or video media need to be connected to MF. E.g., both the originating and terminating side. |
| TranscodeMode | 6.2.6.3.5 | The mode of data channel transcoding. |
| SendRecvMode | 6.2.6.3.6 | The send and recv mode of the media stream. |

Table 6.2.6.1-2 specifies data types re-used by the Nimsas_MediaControl service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nimsas_MediaControl service based interface.

Table 6.2.6.1-2: Nimsas_MediaControl re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| DcStream | 3GPP TS 29.571 [16] | Data Channel mapping and configuration information. |
| MdcEndpoint | 3GPP TS 29.571 [16] | The MDC1 and MDC2 Endpoint information. |
| MaxMessageSize | 3GPP TS 29.571 [16] | Maximum SCTP user message size. |
| Mdc2Protocol | 3GPP TS 29.176 [17] | Transport layer protocols for MDC2 interface. |
| MediaId | 3GPP TS 29.571 [16] | IMS media flow identity. |
| MediaProxy | 3GPP TS 29.571 [16] | Media proxy configuration applicable to the media flow. |
| MediaResourceType | 3GPP TS 29.571 [16] | IMS media resource type. |
| ReplaceHttpUrl | 3GPP TS 29.571 [16] | A list of replacement HTTP URL per stream ID. |
| SessionId | 3GPP TS 29.571 [16] | IMS session identity. |
| Uri | 3GPP TS 29.571 [16] | URI |
| BdcUsedBy | 3GPP TS 29.571 [16] | The party using the bootstrap data channel. |
| AppBindingInfo | 3GPP TS 29.571 [16] | The application binding information of the application data channel. |
| QosHintInfo | 6.1.6.2.7 | The QoS hint information of the media. |
| AvatarMedia | 3GPP TS 29.176 [17] | Avatar media specification. |

6.2.6.2	Structured data types

6.2.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

6.2.6.2.2	Type: MediaInstructionData

Table 6.2.6.2.2-1: Definition of type MediaInstructionData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| sessionId | SessionId | M | 1 | The session ID is the identity of the IMS session for which the MediaInstructions applies. |
| mediaInstructionSet | map(MediaInstructions) | M | 1..N | The mediaInstructionSet includes a set of instructions for each media flow to control. The key of the map shall be a (unique) valid JSON string per clause 7 of IETF RFC 8259 [20], with a maximum of 32 characters. |

6.2.6.2.3	Type: MediaInstructions

Table 6.2.6.2.3-1: Definition of type MediaInstructions

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| mediaId | MediaId | M | 1 | The media ID uniquely identifies this media item within the list and used by the producer (IMS AS) and the consumer for referencing purposes. The consumer reuses the media ID it received from the IMS AS for referencing the same media. This field shall be null for instructions related to originating new media or for data channel media components de-multiplexed from a data channel multiplexing media which do not reuse the media ID of that data channel multiplexing media. |
| associatedMediaId | MediaId | C | 0..1 | The identifier of the original multiplexed media stream which is de-multiplexed. It shall be included only when the mediaId is null, mediaInstruction is set to "ORIGINATE" and the media stream identified by the value of this attribute is de-multiplexed. |
| mediaResourceType | MediaResourceType | M | 1 | The mediaResourceType identify the Media Resource types the Media instruction is intended for. |
| mediaInstruction | MediaInstruction | C | 0..1 | It shall be included if the mediaResourceType is set to "DC". When present, mediaInstruction includes instructions to the producer (IMS AS) for handling the media. |
| dcMediaSpecification | DcMediaSpecification | C | 0..1 | dcMediaSpecification includes the description of additional media specification information which are needed for data channel media stream from application layer. It shall be contained if the mediaResourceType is set to "DC". |
| arMediaSpecification | ArMediaSpecification | C | 0..1 | arMediaSpecification includes the description of additional media specification information which are needed for AR communication services from application layer. It shall be contained if the mediamedia is used for AR communication. |
| mediaProcessingUrl | Uri | C | 0..1 | The mediaProcessingURL indicates the address where MF receive service-related media instructions. This field shall be contained in the response when originating a new media. |
| audioVideoReNegotiationInd | AudioVideoReNegotiationInd | C | 0..1 | Represents the audio/video media re-negotiation indication information to connect the UE's audio/video media stream to MF which is required by the application data channel. It shall be included if the mediaResourceType is set to "AUDIO" or "VIDEO". |
| avatarMedia | AvatarMedia | O | 0..1 | Represents the additional media specifications needed for avatar communication services. |

6.2.6.2.4	Type: DcMediaSpecification

Table 6.2.6.2.4-1: Definition of type DcMediaSpecification

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| mediaProxyConfig | MediaProxy | M | 1 | Represents the media proxy configuration on the MF. The value shall be set to "HTTP_PROXY" if the mediaId represents the bootstrap data channel. The value shall be set to "DC_APPLICATION_PROXY" if the mediaId represents the network initiated P2P application data channel. |
| replaceHttpUrls | map(ReplaceHttpUrl) | C | 1..N | Represents a list of replacement HTTP URL per stream ID allocated by the application layer for the specific IMS subscriber when requesting the application list (e.g. graphical user interface) via the MDC1 interface. It shall be included if the mediaId represents a bootstrap data channel and streamId is 0 or 100. The streamId attribute within the ReplaceHttpUrl data type is the key of the map. |
| mdc1EndpointDcsf | MdcEndpoint | C | 0..1 | Represents the MDC1 media endpoint information reserved on DCSF, e.g., the IP address and port number of DCSF. It shall be included in the request of the media instruction if the mediaId represents a bootstrap data channel. (NOTE) |
| mdc1EndpointMf | MdcEndpoint | C | 0..1 | Represents the negotiated MDC1 media endpoint information on MF, e.g., the IP address and port number of DC capable MF. It shall be included in the request of the media instruction if the mediaId represents a bootstrap data channel. (NOTE) |
| mdc2EndpointInfo | Mdc2EndpointInfo | C | 0..1 | Represents the negotiated MDC2 media endpoint information, e.g., the protocol stack, IP address and port number. It shall be included if in the request of the media instruction the mediaId represents anapplication data channel. |
| streams | map(DcStream) | M | 1..N | Represents the data channel mapping and configuration information when originating/terminating data channel media flows on the Mb interface. The streamId attribute within the DcStream data type is the key of the map. |
| bdcUsedby | BdcUsedBy | C | 0..1 | Represents the party using the bootstrap data channel. It shall be contained when the mediaId in the data type MediaInstructions represents bootstrap data channel and the streamId sets to 100 and 110. |
| appBindingInfo | AppBindingInfo | C | 0..1 | Represents the application binding information of the application data channel. It shall be contained when the mediaId in the data type MediaInstrutions represents application data channel. |
| qosHintInfo | QosHintInfo | O | 0..1 | Represents the QoS information that is provided within the SDP media level attribute "a=3gpp-qos-hint". |
| interworkingInfo | InterworkingInstruction | O | 0..1 | Represents how to transcode the data channel media via MF. |
| dcIwkInd | boolean | O | 0..1 | Indicates the DC media is for DC interworking via DC AS. This attribute is set to "true" if the DC media is for DC interworking via DC AS. If the attribute is not present it means it is "false". |
| NOTE: The MDC1 interface shall use "TCP/TLS/HTTP" protocol stack and the corresponding IP, TCP, TLS and HTTP related attributes in MdcEndpoint should be included. |  |  |  |  |

6.2.6.2.5	Type: ArMediaSpecification

Table 6.2.6.2.5-1: Definition of type ArMediaSpecification

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| mediaProcessingSpec | string | M | 1 | It specifies how the AR media stream should be processed. It indicates how the MF assists in the AR media rendering function. |

6.2.6.2.6	Type: Mdc2EndpointInfo

Table 6.2.6.2.6-1: Definition of type Mdc2EndpointInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| mdc2Protocol | Mdc2Protocol | C | 0..1 | Represents the transport layer protocol or protocol stack for MDC2 interface when the "HTTP_PROXY" media proxy configuration is used in MF. It shall be included if the value of the mediaProxyConfig within the data type DcMediaSpecification is set to "HTTP_PROXY". |
| mdc2EndpointDcAs | MdcEndpoint | C | 0..1 | Represents the negotiated MDC2 media endpoint information on DC AS. It shall be included in the POST request message for the media control. The attributes "tlsId", "sctpPort" and "fingerprint" within the MdcEndpoint shall be present when the value of the mediaProxyConfig within the DcMediaSpecification is set to "UDP_PROXY". (NOTE) |
| mdc2EndpointMf | MdcEndpoint | C | 0..1 | Represents the negotiated MDC2 media endpoint information on MF. It shall be included in the successful POST response message for the media control API. The attributes "tlsId", "sctpPort" and "fingerprint" within the MdcEndpoint shall not be present when the value of the mediaProxyConfig within the data type DcMediaSpecification is set to "UDP_PROXY". (NOTE) |
| NOTE: When the mdc2Protocol is set to "UDP/DTLS/SCTP", "TCP/TLS" or "SCTP/DTLS", the attributes "tlsId" and "fingerprint" within the data type MdcEndpoint shall be included, and the "securitySetup" may be included. When the value of mdc2Protocol is set to " UDP/DTLS/SCTP", the attribute "sctpPort" within the data type MdcEndpoint shall be included. |  |  |  |  |

6.2.6.2.7	Type: AudioVideoReNegotiationInd

Table 6.2.6.2.7-1: Definition of type AudioVideoReNegotiationInd

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| reNegotiation | boolean | M | 1 | When present, the IE shall indicate whether the audio or video media re-negotiation to connect the UE’s media stream to MF is required by the application data channel. The media type will be represented by the value of the mediaResourceType. - true: media re-negotiation to connect the UE's audio or video media stream to MF is required. - false: media re-negotiation to connect the UE's audio or video media stream to MF is not required. |
| mediaConnSide | MediaConnSide | C | 0..1 | Represent the audio or video media connection side that the application data channel required. It shall be included if the reNegotiation present and the attribute value is set to true. |
| mediaDirection | SendRecvMode | O | 0..1 | Represents the media direction of the audio/video media. |

6.2.6.2.8	Type: InterworkingInstruction

Table 6.2.6.2.8-1: Definition of type InterworkingInstruction

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| transcodeMode | TranscodeMode | M | 1 | Indicates the mode of the media transcoding. |
| addTransInfo | string | O | 0..1 | Additional information on how to transcode the data channel media. |

6.2.6.3	Simple data types and enumerations

6.2.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

6.2.6.3.2	Simple data types

The simple data types defined in table 6.2.6.3.2-1 shall be supported.

Table 6.2.6.3.2-1: Simple data types

| Type Name | Type Definition | Description |
| --- | --- | --- |

6.2.6.3.3	Enumeration: MediaInstruction

The enumeration MediaInstruction represents the instructions to the producer (IMS AS) for handling the media. It shall comply with the provisions defined in table 6.2.6.3.3-1.

Table 6.2.6.3.3-1: Enumeration MediaInstruction

| Enumeration value | Description |
| --- | --- |
| "TERMINATE_MEDIA" | Terminate the offered media descriptor of the UE in the media resource. |
| "ORIGINATE_MEDIA" | Originate and offer a media descriptor from the mediaResource to the UE. |
| "TERMINATE_AND_ORIGINATE_MEDIA" | Terminate the offered media flow in the mediaResource from the UE and originate the same media flow from the mediaResource to the other UE. |
| "UPDATE_MEDIA" | Update a media flow of the mediaResource previously allocated by the instructions "TerminateMedia", "OriginateMedia" and "TerminateAndOriginateMedia". |
| "DELETE_MEDIA" | Delete a media flow of the mediaResource previously allocated by the instructions "TerminateMedia", "OriginateMedia" and "TerminateAndOriginateMedia". |
| "REJECT_MEDIA" | Remove an offered media flow. |

6.2.6.3.4	Enumeration: MediaConnSide

The enumeration MediaConnSide represents the audio or video media connection side. It shall comply with the provisions defined in table 6.2.6.3.4-1.

Table 6.2.6.3.4-1: Enumeration MediaConnSide

| Enumeration value | Description |
| --- | --- |
| "BOTH" | Both the originating and terminating side of the audio or video media stream shall be connected to the MF. |
| "TERMINATING_SIDE" | Only the terminating side of the audio or video media stream are connected to the MF. |
| "ORIGINATING_SIDE" | Only the originating side of the audio or video media stream are connected to the MF. |

6.2.6.3.5	Enumeration: TranscodeMode

The enumeration TranscodeMode represents the media transcode instruction. It shall comply with the provisions defined in table 6.2.6.3.5-1.

Table 6.2.6.3.5-1: Enumeration TranscodeMode

| Enumeration value | Description |
| --- | --- |
| "DC_TO_VIDEO" | Data channel media is to be transcoded to video media. |

6.2.6.3.6	Enumeration: SendRecvMode

The enumeration SendRecvMode represents the audio or video media connection side. It shall comply with the provisions defined in table 6.2.6.3.6-1.

Table 6.2.6.3.6-1: Enumeration SendRecvMode

| Enumeration value | Description |
| --- | --- |
| "SENDRECV" | The media is send and recv. |
| "SENDONLY" | The media is send only. |
| "RECVONLY" | The media is receive only. |

6.2.6.4	Data types describing alternative data types or combinations of data types

None in this release of the specification.

6.2.6.5	Binary data

None in this release of the specification.

6.2.7	Error Handling

6.2.7.1	General

For the Nimsas_MediaControl API, HTTP error responses shall be supported as specified in clause 4.8 of 3GPP TS 29.501 [5]. Protocol errors and application errors specified in table 5.2.7.2-1 of 3GPP TS 29.500 [4] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.2.7.1-1 of 3GPP TS 29.500 [4].

In addition, the requirements in the following clauses are applicable for the Nimsas_MediaControl API.

6.2.7.2	Protocol Errors

No specific procedures for the Nimsas_MediaControl service are specified.

6.2.7.3	Application Errors

The application errors defined for the Nimsas_MediaControl service are listed in table 6.2.7.3-1.

Table 6.2.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| MEDIA_ID_NOT_FOUND | 400 Bad Request | Indicates that the requested mediaId is not found in the IMS AS. |

6.2.8	Feature negotiation

The optional features in table 6.2.8-1 are defined for the Nimsas_MediaControl API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 6.2.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

6.2.9	Security

As indicated in 3GPP TS 33.501 [8] and 3GPP TS 29.500 [4], the access to the Nimsas_MediaControl API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [9]), based on local configuration, using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [10]) plays the role of the authorization server.

If OAuth2 is used, an NF Service Consumer, prior to consuming services offered by the Nimsas_MediaControl API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [10], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Nimsas_MediaControl service.

The Nimsas_MediaControl API defines a single scope " nimsas-mc " for the entire service, and it does not define any additional scopes at resource or operation level.

6.2.10	HTTP redirection

An HTTP request may be redirected to a different IMS AS service instance when using direct or indirect communications (see 3GPP TS 29.500 [4]).

An SCP that reselects a different IMS AS producer instance will return the NF Instance ID of the new IMS AS producer instance in the 3gpp-Sbi-Producer-Id header, as specified in clause 6.10.3.4 of 3GPP TS 29.500 [4].

If an IMS AS redirects a service request to a different IMS AS using an 307 Temporary Redirect or 308 Permanent Redirect status code, the identity of the new IMS AS towards which the service request is redirected shall be indicated in the 3gpp-Sbi-Target-Nf-Id header of the 307 Temporary Redirect or 308 Permanent Redirect response as specified in clause 6.10.9.1 of 3GPP TS 29.500 [4].

6.3	Nimsas_ImsSessionManagement Service API

6.3.1	API URI

The Nimsas_ImsSessionManagement service shall use the Nimsas_ImsSessionManagement API.

The API URI of the Nimsas_ImsSessionManagement API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests from the NF service consumer towards the NF service producer shall have the Resource URI structure defined in clause 4.4.1 of 3GPP TS 29.501 [5], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [5].

-	The <apiName> shall be "nimsas-ism".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 6.3.3.

6.3.2	Usage of HTTP

6.3.2.1	General

HTTP/2, IETF RFC 7540 [11], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

The OpenAPI [6] specification of HTTP messages and content bodies for the Nimsas_SessionEventControl API is contained in Annex A.

6.3.2.2	HTTP standard headers

6.3.2.2.1	General

See clause 5.2.2 of 3GPP TS 29.500 [4] for the usage of HTTP standard headers.

6.3.2.2.2	Content type

JSON, IETF RFC 8259 [12], shall be used as content type of the HTTP bodies specified in the present specification as specified in clause 5.4 of 3GPP TS 29.500 [4]. The use of the JSON format shall be signalled by the content type "application/json".

"Problem Details" JSON object shall be used to indicate additional details of the error in a HTTP response body and shall be signalled by the content type "application/problem+json", as defined in IETF RFC 7807 [13].

6.3.2.3	HTTP custom headers

The mandatory HTTP custom header fields specified in clause 5.2.3.2 of 3GPP TS 29.500 [4] shall be supported, and the optional HTTP custom header fields specified in clause 5.2.3.3 of 3GPP TS 29.500 [4] may be supported.

6.3.3	Resources

6.3.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.3.3.1-1 depicts the resource URIs structure for the Nimsas_ImsSessionManagement service API.

Figure 6.3.3.1-1: Resource URI structure of the Nimsas_ImsSessionManagement API

Table 6.3.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.3.3.1-1: Resources and methods overview

| Resource purpose/name | Resource URI (relative path after API URI) | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| IMS Sessions (Collection) | /ims-sessions | POST | Creates a new Individual IMS Session resource. |
| Individual IMS Session (Document) | /ims-sessions/{sessionId} | PUT | Update an existing "Individual IMS Session" resource. |
|  |  | PATCH | Modify an existing Individual IMS Session resource. |
|  |  | DELETE | Delete an existing Individual IMS Session resource. |
| IMS Session Event Subscriptions (Collection) | /ims-sessions/event-subscriptions | POST | This is a pseudo resource. |

6.3.3.2	Resource: IMS Sessions

6.3.3.2.1	Description

The IMS Sessions Collection resource represents a collection of IMS Sessions created by NF service consumers of Nimsas_ImsSessionManagement service at a given IMS AS. The resource is modelled as Collection resource archetype (see clause C.2 of 3GPP TS 29.501 [5]).

6.3.3.2.2	Resource Definition

Resource URI: {apiRoot}/nimsas-ism/<apiVersion>/ims-sessions

This resource shall support the resource URI variables defined in table 6.1.3.2.2-1.

Table 6.3.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.3.1 |

6.3.3.2.3	Resource Standard Methods

6.3.3.2.3.1	POST

This method shall support the URI query parameters specified in table 6.3.3.2.3.1-1.

Table 6.3.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.3.3.2.3.1-2 and the response data structures and response codes specified in table 6.3.3.2.3.1-3.

Table 6.3.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ImsSessionInfo | M | 1 | Creates a new Individual IMS Session resource |

Table 6.3.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ImsSessionInfo | M | 1 | 201 Created | The creation of an Individual IMS Session resource is confirmed and a representation of that resource is returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| NOTE 1: The manadatory HTTP error status code for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.3.3.2.3.1-4: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/nimsas-ism/<apiVersion>/ims-sessions/{sessionId}. |

Table 6.3.3.2.3.1-5: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative IMS AS (service) instance. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target IMS AS instance ID towards which the request is redirected. |

Table 6.3.3.2.3.1-6: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative IMS AS (service) instance. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target IMS AS instance ID towards which the request is redirected. |

6.3.3.2.4	Resource Custom Operations

None.

6.3.3.3	Resource: Individual IMS Session

6.3.3.3.1	Description

The Individual IMS Session resource represents an individual IMS session created by the NF service consumers of Nimsas_ImsSessionManagement service at a given IMS AS. This resource is modelled as the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [5]).

6.3.3.3.2	Resource Definition

Resource URI: {apiRoot}/nimsas_ism/<apiVersion>/ims-sessions/{sessionId}

The <apiVersion> shall be set as described in clause 6.3.1.

This resource shall support the resource URI variables defined in table 6.3.3.3.2-1.

Table 6.3.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.3.1 |
| sessionId | string | Identifies an individual IMS session to the Nimsas_ImsSessionManagement service. |

6.3.3.3.3	Resource Standard Methods

6.3.3.3.3.1	PATCH

This method shall support the URI query parameters specified in table 6.3.3.3.3.1-1.

Table 6.3.3.3.3.1-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.3.3.3.3.1-2 and the response data structures and response codes specified in table 6.3.3.3.3.1-3.

Table 6.3.3.3.3.1-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(PatchItem) | M | 1..N | Document describes the modification(s) to an Individual IMS Session resource. |

Table 6.3.3.3.3.1-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ImsSessionInfo | M | 1..N | 200 OK | Represents a successful update on the IMS session. |
| n/a |  |  | 204 No Content |  |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| ProblemDetails | O | 0..1 | 409 Conflict | Indicates the creation of media context has failed due to application error. The "cause" attribute may be used to indicate one of the following application errors: - MEDIA_CORRELATION_ID_CONFLICT, if the media to be created has the identical mediaCorrelationId with one already created media. |
| ProblemDetails | O | 0..1 | 404 Not Found | Indicates the modification of IMS session has failed due to application error. The "cause" attribute may be used to indicate one of the following application errors: - SESSION_NOT_FOUND |
| NOTE 1: The manadatory HTTP error status code for the PATCH method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.3.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative IMS AS (service) instance. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target IMS AS instance ID towards which the request is redirected. |

Table 6.3.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative IMS AS (service) instance. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target IMS AS instance ID towards which the request is redirected. |

6.3.3.3.3.2	DELETE

This method shall support the URI query parameters specified in table 6.3.3.3.3.2-1.

Table 6.3.3.3.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.3.3.3.3.2-2 and the response data structures and response codes specified in table 6.3.3.3.3.2-3.

Table 6.3.3.3.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.3.3.3.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No content |  |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| Problem Details | O | 0..1 | 404 Not Found | Indicates the deletion of the IMS session has failed due to application error. The "cause" attribute may be used to indicate one of the following application errors: - SESSION_NOT_FOUND |
| NOTE 1: The manadatory HTTP error status code for the DELETE method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.3.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative IMS AS (service) instance. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target IMS AS instance ID towards which the request is redirected. |

Table 6.3.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative IMS AS (service) instance. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target IMS AS instance ID towards which the request is redirected. |

6.3.3.3.3.3	PUT

This method shall support the URI query parameters specified in table 6.3.3.3.3.3-1.

Table 6.3.3.3.3.3-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.3.3.3.3.3-2 and the response data structures and response codes specified in table 6.3.3.3.3.3-3.

Table 6.3.3.3.3.3-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ImsSessionInfo | M | 1..N | Parameters to update the "Individual IMS Session" resource. |

Table 6.3.3.3.3.3-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ImsSessionInfo | M | 1..N | 200 OK | Successful response. The "Individual IMS Session" resource is successfully updated and the representation of that resource is returned. |
| n/a |  |  | 204 No Content | Successful response. The "Individual IMS Session" resource is successfully updated and no content is returned in the response body. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| ProblemDetails | O | 0..1 | 404 Not Found | Indicates the modification of Individual IMS Event Subscription has failed due to application error. The “cause” attribute may be used to indicate one of the following application errors: - SUBSCRIPTION_NOT_FOUND |
| NOTE 1: The manadatory HTTP error status code for the PUT method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.3.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative IMS AS (service) instance. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target IMS AS instance ID towards which the request is redirected. |

Table 6.3.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative IMS AS (service) instance. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target IMS AS instance ID towards which the request is redirected. |

6.3.3.3.4	Resource Custom Operations

None.

6.3.4	Custom Operations without associated resources

None in this release of specification.

6.3.5	Notifications

6.3.5.1	General

Notifications shall comply to clause 6.2 of 3GPP TS 29.500 [4] and clause 4.6.2.3 of 3GPP TS 29.501 [5].

Table 6.3.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| IMS Session Event Notification | {notifUri} | POST | Report the IMS session event observed. |

6.3.5.2	IMS Session Event Notification

6.3.5.2.1	Description

The IMS Session Event Notification is used by the IMS AS to report the observed IMS Session Event to notification endpoints.

6.3.5.2.2	Target URI

The Callback URI "{notifUri}" shall be used with the callback URI variables defined in table 6.3.5.2.2-1.

Table 6.3.5.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notifUri | The notification URI of the Service Consumer (i.e., NEF, trusted AF) to receive the IMS session events. |

6.3.5.2.3	Standard Methods

6.3.5.2.3.1	POST

This method shall support the request data structures specified in table 6.3.5.2.3.1-1 and the response data structures and response codes specified in table 6.3.5.2.3.1-2.

Table 6.3.5.2.3.1-1: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ImsSessionEventNotification | M | 1 | The IMS session event notification to notification endpoints. |

Table 6.3.5.2.3.1-2: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | This case represents a successful notification of the event. |
| ProblemDetails | O | 0..1 | 404 Not Found | Indicates the session event notification has failed due to application error. The "cause" attribute may be used to indicate one of the following application errors: - NOTIFICATION_CORRELATION_NOT_FOUND, e.g. if the NF Service Consumer (e.g. NEF, trusted AF) considers the notification correlation ID is not recognized; - NOTIFICATION_URI_NOT_FOUND, if the NF Service Consumer (e.g. NEF, trusted AF) considers the notification URI is not recognized. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.3.5.2.3-3: Headers supported by the 307 Response Code on this endpoint

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | A URI pointing to the endpoint of the NF service consumer instance to which the request should be sent. For the case, when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF instance ID towards which the notification is redirected |

Table 6.3.5.2.3-4: Headers supported by the 308 Response Code on this endpoint

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | A URI pointing to the endpoint of the NF service consumer instance to which the request should be sent. For the case, when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF instance ID towards which the notification is redirected |

6.3.6	Data Model

6.3.6.1	General

This clause specifies the application data model supported by the API.

Table 6.3.6.1-1 specifies the data types defined for the Nimsas_ImsSessionManagement service based interface protocol.

Table 6.3.6.1-1: Nimsas_ImsSessionManagement specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| ImsSessionInfo | 6.3.6.2.2 | The IMS session information and the media list used in the IMS session. |
| DcMediaPara | 6.3.6.2.5 | The parameters of the IMS DC media. |
| ImsSessionEventNotification | 6.3.6.2.6 | The IMS Session event notification. |
| MediaInfoExternal | 6.3.6.2.4 | The media information used in the external created or updated IMS session. |
| SessionInfoSM | 6.3.6.2.3 | The session information to be used. |
| TargetIdInfo | 6.3.6.2.7 | The public identity(ies) of the IMS Subscriber the DC is targeted. |
| DcType | 6.3.6.3.3 | The type of data channel. |
| AdcType | 6.3.6.3.4 | The type of the application data channel. |
| ImsSessionEventType | 6.3.6.3.6 | The event type of IMS session event. |

Table 6.3.6.1-2 specifies data types re-used by the Nimsas_ImsSessionManagement service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nimsas_ImsSessionManagement service based interface.

Table 6.3.6.1-2: Nimsas_ImsSessionManagement re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| AppBindingInfo | 3GPP TS 29.571 [16] | The application binding information of the application data channel. |
| ImsPublicId | 3GPP TS 29.562 [15] | IMS Public Identity. |
| MediaType | 6.1.6.3.4 | The media type used in the IMS session. |
| SessionId | 3GPP TS 29.571 [16] | IMS Session Identity. |
| Mdc2EndpointInfo | 6.2.6.2.6 | The MDC2 media endpoint information. |
| MediaInfo | 6.1.6.2.5 | The media information related to the notification event. |
| Uri | 3GPP TS 29.571 [16] | URI. |

6.3.6.2	Structured data types

6.3.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

6.3.6.2.2	Type: ImsSessionInfo

Table 6.3.6.2.2-1: Definition of type ImsSessionInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| sessionId | SessionId | C | 0..1 | The session ID is the identity of the IMS session. This IE contains the service producer generated identity which uniquely identifies the IMS session. It shall not be contained when creating of a new IMS session. Otherwise, it shall be contained. |
| sessionInfo | SessionInfoSM | M | 1 | The sessionInfo is the IMS session related information. |
| mediaInfoSet | array(MediaInfoExternal) | M | 1..N | Media info set includes a list of media related information. |
| notifUri | Uri | C | 0..1 | This IE shall only be present if the NF consumer intends to receive IMS session events for the served IMS subscriber. When present, it shall contain the URI of the NF consumer to receive IMS session event notification. |
| notifCorrelationId | string | C | 0..1 | This IE shall be present together with the notifUri, if available. When present, it shall contain the correlation ID to associate the IMS session event notification to be reported. |

6.3.6.2.3	Type: SessionInfoSM

Table 6.3.6.2.3-1: Definition of type SessionInfoSM

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| callingId | ImsPublicId | M | 1 | The public identity of the calling IMS subscriber or the identity of the consumer NF, e.g. DC AS. |
| calledId | ImsPublicId | M | 1 | The public identity of the called IMS Subscriber. |

6.3.6.2.4	Type: MediaInfoExternal

Table 6.3.6.2.4-1: Definition of type MediaInfoExternal

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| mediaType | MediaType | M | 1 | The media type used in the session. In this release, the mediaType can only set to "DC". |
| mediaCorrelationId | string | M | 1 | The media correlation ID is the identity of the media. |
| dcMediaParas | array(DcMediaPara) | C | 1..N | The information of the created data channel media. It shall be contained when the attribute "mediaType" is set to "DC". |

6.3.6.2.5	Type: DcMediaPara

Table 6.3.6.2.5-1: Definition of type DcMediaPara

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| dcType | DcType | M | 1 | The data channel type of the data channel. |
| adcType | AdcType | C | 0..1 | The type of application data type. This IE shall be contained when the attribute "dcType" is set to "ADC". |
| appBindingInfo | AppBindingInfo | C | 0..1 | Represents the application binding information of the application data channel. It shall be contained when the attribute "dcType" is set to "ADC". |
| targetIds | array(TargetIdInfo) | C | 1..N | Representes the public identity(ies) of the IMS Subscriber the DC is targeted. It shall be contained when the attribute "dcType" is set to "ADC". |
| targetId | ImsPublicId | C | 0..1 | the public identity of the IMS Subscriber the BDC is targeted. It shall be contained when the attribute "dcType" is set to "BDC". |
| mdc2EndpointInfo | Mdc2EndpointInfo | C | 0..1 | Represents the negotiated MDC2 media endpoint information, e.g., the protocol stack, IP address and port number. It shall be contained when the attribute "dcType" is set to "ADC" and adcType set to "P2A" or "P2A2P". |

6.3.6.2.6	Type: ImsSessionEventNotification

Table 6.3.6.2.6-1: Definition of type ImsSessionEventNotification

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| sessionId | SessionId | M | 1 | The session ID is the identity of the IMS session. This IE contains the value of the sessionId attribute provided during the IMS session creation within the ImsSessionInfo data type. |
| eventType | ImsSessionEventType | M | 1 | This IE indicates the type of IMS session event. |
| notifCorrelationId | string | M | 1 | This IE indicates the correlation ID used to associate the IMS session event notification to be reported. |
| mediaInfoSet | array(MediaInfoExternal) | O | 1..N | Media info set includes a list of media related information. |

6.3.6.2.7	Type: TargetIdInfo

Table 6.3.6.2.7-1: Definition of type TargetIdInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| callingIdentity | ImsPublicId | C | 0..1 | The IMS public identity of the calling IMS subscriber. It shall be contained when the attribute adcType set to "P2P" or "P2A2P". (NOTE) |
| calledIdentity | ImsPublicId | C | 0..1 | The IMS public identity of the called IMS Subscriber. It shall be contained when the attribute adcType set to "P2P" or "P2A2P". (NOTE) |
| NOTE: One of the callingIdentity or calledIdentity shall be contained when the attribute adcType set to "P2A". |  |  |  |  |

6.3.6.3	Simple data types and enumerations

6.3.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

6.3.6.3.2	Simple data types

The simple data types defined in table 6.3.6.3.2-1 shall be supported.

Table 6.3.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

6.3.6.3.3	Enumeration: DcType

The enumeration DcType represents the type of data channel. It shall comply with the provisions defined in table 6.3.6.3.3-1.

Table 6.3.6.3.3-1: Enumeration DcType

| Enumeration value | Description |
| --- | --- |
| "BDC" | Bootstrap data channel. |
| "ADC" | Application data channel. |

6.3.6.3.4	Enumeration: AdcType

The enumeration AdcType represents the type of the application data channel. It shall comply with the provisions defined in table 6.3.6.3.4-1.

Table 6.3.6.3.4-1: Enumeration AdcType

| Enumeration value | Description |
| --- | --- |
| "P2P" | P2P application data channel |
| "P2A" | P2A application data channel |
| "P2A2P" | P2A2P application data channel |

6.3.6.3.5	Void

6.3.6.3.6	Enumeration: ImsSessionEventType

The enumeration ImsSessionEventType represents the IMS session event type for which the notification is generated. It shall comply with the provisions defined in table 6.3.6.3.6-1.

Table 6.3.6.3.6-1: Enumeration ImsSessionEventType

| Enumeration value | Description |
| --- | --- |
| "SESSION_ESTABLISHMENT_SUCCESS" | Session establishment success event. |
| "SESSION_ESTABLISHMENT_FAILURE" | Session establishment failure event. |
| "MEDIA_CHANGE_SUCCESS" | Media change success event. |
| "MEDIA_CHANGE_FAILURE" | Media change failure event. |
| "SESSION_TERMINATION" | Session termination event. |

6.3.6.4	Data types describing alternative data types or combinations of data types

None in this release of the specification.

6.3.6.5	Binary data

None in this release of the specification.

6.3.7	Error Handling

6.3.7.1	General

For the Nimsas_ImsSessionManagement API, HTTP error responses shall be supported as specified in clause 4.8 of 3GPP TS 29.501 [5]. Protocol errors and application errors specified in table 5.2.7.2-1 of 3GPP TS 29.500 [4] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.2.7.1-1 of 3GPP TS 29.500 [4].

In addition, the requirements in the following clauses are applicable for the Nimsas_ImsSessionManagement API.

6.3.7.2	Protocol Errors

No specific procedures for the Nimsas_ImsSessionManagement service are specified.

6.3.7.3	Application Errors

The application errors defined for the Nimsas_ImsSessionManagement service are listed in Table 6.3.7.3-1.

Table 6.3.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| SESSION_NOT_FOUND | 404 Not Found | The service user is not served by this DCSF, e.g. no data channel configurations for this service user is found. |
| NOTIFICATION_CORRELATION_NOT_FOUND | 404 Not Found | The NF Service Consumer (e.g. NEF, trusted AF) considers the notification correlation ID is not recognized. |
| NOTIFICATION_URI_NOT_FOUND | 404 Not Found | The NF Service Consumer (e.g. NEF, trusted AF) considers the notification URI is not recognized. |
| MEDIA_CORRELATION_ID_CONFLICT | 409 Conflict | Indicates that a new media to be created has the identical mediaCorrelationId already assigned to one created media. |

6.3.8	Feature negotiation

The optional features in table 6.3.8-1 are defined for the Nimsas_ImsSessionManagement API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 6.3.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

6.3.9	Security

As indicated in 3GPP TS 33.501 [8] and 3GPP TS 29.500 [4], the access to the Nimsas_ImsSessionManagement API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [9]), based on local configuration, using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [10]) plays the role of the authorization server.

If OAuth2 is used, an NF Service Consumer, prior to consuming services offered by the Nimsas_ImsSessionManagement API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [10], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Nimsas_ImsSessionManagement service.

The Nimsas_ImsSessionManagement API defines a single scope "nimsas-imssessionmanagement" for the entire service, and it does not define any additional scopes at resource or operation level.

6.3.10	HTTP redirection

An HTTP request may be redirected to a different IMS AS service instance when using direct or indirect communications (see 3GPP TS 29.500 [4]).

An SCP that reselects a different NF provider (e.g. IMS AS) instance will return the NF Instance ID of the new NF provider instance in the 3gpp-Sbi-Producer-Id header, as specified in clause 6.10.3.4 of 3GPP TS 29.500 [4].

If an NF provider (e.g. IMS AS) redirects a service request to a different NF provider using a 307 Temporary Redirect or 308 Permanent Redirect status code, the identity of the new NF provider towards which the service request is redirected shall be indicated in the 3gpp-Sbi-Target-Nf-Id header of the 307 Temporary Redirect or 308 Permanent Redirect response as specified in clause 6.10.9.1 of 3GPP TS 29.500 [4].

6.4	Nimsas_ImsEE Service API

6.4.1	API URI

The Nimsas_ImsEE service shall use the Nimsas_ImsEE API.

The API URI of the Nimsas_ImsEE API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests from the NF service consumer towards the NF service producer shall have the Resource URI structure defined in clause 4.4.1 of 3GPP TS 29.501 [5], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [5].

-	The <apiName> shall be "nimsas-ee".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 6.4.3.

6.4.2	Usage of HTTP

6.4.2.1	General

HTTP/2, IETF RFC 7540 [11], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

The OpenAPI [6] specification of HTTP messages and content bodies for the Nimsas_ImsEE API is contained in Annex A.

6.4.2.2	HTTP standard headers

6.4.2.2.1	General

See clause 5.2.2 of 3GPP TS 29.500 [4] for the usage of HTTP standard headers.

6.4.2.2.2	Content type

JSON, IETF RFC 8259 [12], shall be used as content type of the HTTP bodies specified in the present specification as specified in clause 5.4 of 3GPP TS 29.500 [4]. The use of the JSON format shall be signalled by the content type "application/json".

"Problem Details" JSON object shall be used to indicate additional details of the error in a HTTP response body and shall be signalled by the content type "application/problem+json", as defined in IETF RFC 7807 [13].

6.4.2.3	HTTP custom headers

The mandatory HTTP custom header fields specified in clause 5.2.3.2 of 3GPP TS 29.500 [4] shall be supported, and the optional HTTP custom header fields specified in clause 5.2.3.3 of 3GPP TS 29.500 [4] may be supported.

6.4.3	Resources

6.4.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.4.3.1-1 depicts the resource URIs structure for the Nimsas_EE service API.

Figure 6.4.3.1-1: Resource URI structure of the Nimsas_ImsEE API

Table 6.4.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.4.3.1-1: Resources and methods overview

| Resource purpose/name | Resource URI (relative path after API URI) | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| IMS Event Subscriptions (Collection) | /subscriptions | POST | Creates a new Individual IMS Event Subscription resource. |
| Individual IMS Event Subscription (Document) | /subscriptions/{subscriptionId} | GET | Get the information of a subscription identified by {subscriptionId}. |
|  |  | PUT | Updates an existing Individual IMS Event Subscription resource identified by {subscriptionId}. |
|  |  | PATCH | Updates an existing Individual IMS Event Subscription resource identified by {subscriptionId}. |
|  |  | DELETE | Delete an existing Individual IMS Subscription resource identified by {subscriptionId}. |

6.4.3.2	Resource: IMS Event Subscriptions

6.4.3.2.1	Description

The IMS Event Subscriptions resource represents all subscriptions to the Nimsas_ImsEE service at a given IMS AS. The resource is modelled as Collection resource archetype (see clause C.2 of 3GPP TS 29.501 [5]). The resource allows an NF service consumer to create a new Individual IMS Event Subscription resource.

6.4.3.2.2	Resource Definition

Resource URI: {apiRoot}/nimsas-ee/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in table 6.1.3.2.2-1.

Table 6.4.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.4.1 |

6.4.3.2.3	Resource Standard Methods

6.4.3.2.3.1	POST

This method shall support the URI query parameters specified in table 6.4.3.2.3.1-1.

Table 6.4.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.4.3.2.3.1-2 and the response data structures and response codes specified in table 6.4.3.2.3.1-3.

Table 6.4.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ImsEventsSubscription | M | 1 | Creates a new Individual IMS Event Subscription resource. |

Table 6.4.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ImsEventsSubscriptionCreated | M | 1 | 201 Created | The creation of an Individual IMS Event Subscription resource is confirmed and a representation of that resource is returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| ProblemDetails | O | 0..1 | 403 Forbidden | Indicates the creation of Individual IMS Event Subscription has failed due to application error. The "cause" attribute may be used to indicate one of the following application errors: - OPERATION_NOT_ALLOWED |
| NOTE 1: The manadatory HTTP error status code for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.4.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/nimsas-ee/<apiVersion>/subscriptions/{subscriptionId}. |

Table 6.4.3.2.3.1-5: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative IMS AS (service) instance. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target IMS AS instance ID towards which the request is redirected. |

Table 6.4.3.2.3.1-6: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative IMS AS (service) instance. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target IMS AS instance ID towards which the request is redirected. |

6.4.3.2.4	Resource Custom Operations

None.

6.4.3.3	Resource: Individual IMS Event Subscription

6.4.3.3.1	Description

The Individual IMS Event Subscription resource represents an individual IMS event subscription created by the NF service consumers of Nimsas_ImsEE service at a given IMS AS. This resource is modelled as the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [5]).

6.4.3.3.2	Resource Definition

Resource URI: {apiRoot}/nimsas-ee/<apiVersion>/subscriptions/{subscriptionId}

The <apiVersion> shall be set as described in clause 6.4.1.

This resource shall support the resource URI variables defined in table 6.4.3.3.2-1.

Table 6.4.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.4.1 |
| subscriptionId | string | Identifies an individual IMS event subscription to the Nimsas_ImsEE service. |

6.4.3.3.3	Resource Standard Methods

6.4.3.3.3.1	GET

This method shall support the URI query parameters specified in table 6.4.3.3.3.1-1.

Table 6.4.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.4.3.3.3.1-2 and the response data structures and response codes specified in table 6.4.3.3.3.1-3.

Table 6.4.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.4.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ImsEventsSubscription | M | 1..N | 200 OK | A response body containing representation of the Individual IMS Event Subcription resource. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| ProblemDetails | O | 0..1 | 404 Not Found | Indicates the information retrieval of Individual IMS Event Subscription has failed due to application error. The "cause" attribute may be used to indicate one of the following application errors: - SUBSCRIPTION_NOT_FOUND |
| ProblemDetails | O | 0..1 | 403 Forbidden | Indicates the information retrieval of Individual IMS Event Subscription has failed due to application error. The "cause" attribute may be used to indicate one of the following application errors: - OPERATION_NOT_ALLOWED |
| NOTE 1: The manadatory HTTP error status code for the GET method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.4.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative IMS AS (service) instance. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target IMS AS instance ID towards which the request is redirected. |

Table 6.4.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative IMS AS (service) instance. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target IMS AS instance ID towards which the request is redirected. |

6.4.3.3.3.2	PUT

This method shall support the URI query parameters specified in table 6.4.3.3.3.2-1.

Table 6.4.3.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.4.3.3.3.2-2 and the response data structures and response codes specified in table 6.4.3.3.3.2-3.

Table 6.4.3.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ImsEventsSubscription | M | 1..N | Parameters to replace a subscription to IMS Event Subscription resource. |

Table 6.4.3.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ImsEventsSubscriptionCreated | M | 1..N | 200 OK | Represents a successful update on the subscription and the representation of that resource is returned. |
| n/a |  |  | 204 No Content |  |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| ProblemDetails | O | 0..1 | 404 Not Found | Indicates the modification of Individual IMS Event Subscription has failed due to application error. The "cause" attribute may be used to indicate one of the following application errors: - SUBSCRIPTION_NOT_FOUND |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate one of the following application errors: - OPERATION_NOT_ALLOWED |
| NOTE 1: The manadatory HTTP error status code for the PUT method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.4.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative IMS AS (service) instance. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target IMS AS instance ID towards which the request is redirected. |

Table 6.4.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative IMS AS (service) instance. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target IMS AS instance ID towards which the request is redirected. |

6.4.3.3.3.3	PATCH

This method shall support the URI query parameters specified in table 6.4.3.3.3.3-1.

Table 6.4.3.3.3.1-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.4.3.3.3.3-2 and the response data structures and response codes specified in table 6.4.3.3.3.3-3.

Table 6.4.3.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(PatchItem) | M | 1..N | Parameters to replace a subscription to IMS Event Subscription resource. |

Table 6.4.3.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ImsEventsSubscriptionCreated | M | 1..N | 200 OK | Represents a successful update on the subscription and the representation of that resource is returned. |
| n/a |  |  | 204 No Content |  |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| ProblemDetails | O | 0..1 | 404 Not Found | Indicates the modification of Individual IMS Event Subscription has failed due to application error. The "cause" attribute may be used to indicate one of the following application errors: - SUBSCRIPTION_NOT_FOUND |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate one of the following application errors: - OPERATION_NOT_ALLOWED |
| NOTE 1: The manadatory HTTP error status code for the PATCH method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.4.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative IMS AS (service) instance. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target IMS AS instance ID towards which the request is redirected. |

Table 6.4.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative IMS AS (service) instance. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target IMS AS instance ID towards which the request is redirected. |

6.4.3.3.3.4	DELETE

This method shall support the URI query parameters specified in table 6.4.3.3.3.4-1.

Table 6.4.3.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.4.3.3.3.4-2 and the response data structures and response codes specified in table 6.4.3.3.3.4-3.

Table 6.4.3.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.4.3.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No content |  |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| Problem Details | O | 0..1 | 404 Not Found | Indicates the deletion of the Invidudal IMS session has failed due to application error. The “cause” attribute may be used to indicate one of the following application errors: - SUBSCRIPTION_NOT_FOUND |
| NOTE 1: The manadatory HTTP error status code for the DELETE method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.4.3.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative IMS AS (service) instance. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target IMS AS instance ID towards which the request is redirected. |

Table 6.4.3.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative IMS AS (service) instance. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target IMS AS instance ID towards which the request is redirected. |

6.4.3.3.4	Resource Custom Operations

None.

6.4.4	Custom Operations without associated resources

None in this release of specification.

6.4.5	Notifications

6.4.5.1	General

Notifications shall comply with clause 6.2 of 3GPP TS 29.500 [4] and clause 4.6.2.3 of 3GPP TS 29.501 [5].

Table 6.4.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Event Notification | {notificationURI} | POST | Reports one or several observed Events. |

6.4.5.2	Event Notification

6.4.5.2.1	Description

The Event Notification is used by the IMS AS to report one or several observed Events to an NF service consumer that has subscribed to such Notifications.

6.4.5.2.2	Target URI

The Callback URI {notificationURI} shall be used with the callback URI variables defined in table 6.4.5.1-1.

Table 6.4.5.2.2-1: Callback URI variables

| Name | Data type | Definition |
| --- | --- | --- |
| notificationURI | Uri | The Notification Uri as assigned within the Individual IMS Event Subscription and described within the ImsEventsSubscription type. |

6.4.5.2.3	Standard Methods

6.4.5.2.3.1	POST

This method shall support the URI query parameters specified in table 6.4.5.2.3-1.

Table 6.4.5.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 6.4.5.2.3-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(ImsEventsNotification) | M | 1..N | Provides Information about observed IMS Events. |

Table 6.4.5.2.3-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The receipt of the Notification is acknowledged. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection, during the event notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative NF consumer (service) instance where the notification should be sent. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection, during the event notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative NF consumer (service) instance where the notification should be sent. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 6.4.5.2.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative NF consumer (service) instance towards which the notification should be redirected. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF consumer (service) instance towards which the notification request is redirected. |

Table 6.4.5.2.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative NF consumer (service) instance towards which the notification should be redirected. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF consumer (service) instance towards which the notification request is redirected. |

6.4.6	Data Model

6.4.6.1	General

This clause specifies the application data model supported by the API.

Table 6.4.6.1-1 specifies the data types defined for the Nimsas_ImsEE service based interface protocol.

Table 6.4.6.1-1: Nimsas_ImsEE specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| ImsEventsSubscription | 6.4.6.2.2 | Represents an Individual IMS Event Subscription resource. |
| ImsEventsNotification | 6.4.6.2.4 | The events notification. |
| EventNotification | 6.4.6.2.5 | The notification information for a specific event. |
| ImsEventsSubscriptionCreated | 6.4.6.2.10 | Created Individual IMS Event Subscription |

Table 6.4.6.1-2 specifies data types re-used by the Nimsas_ImsEE service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nimsas_ImsEE service based interface.

Table 6.4.6.1-2: Nimsas_ImsEE re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| DateTime | 3GPP TS 29.571 [16] |  |
| ImsEvent | 3GPP TS 29.571 [16] | The IMS events |
| ImsEventConfiguration | 3GPP TS 29.571 [16] | The IMS event configuration info. |
| ImsPublicId | 3GPP TS 29.562 [15] | IMS Public Identity. |
| ImsEventReport | 3GPP TS 29.571 [16] | The IMS event report. |
| ImsEventReportInfo | 3GPP TS 29.571 [16] | The IMS event report specific information. |
| ImsReportingOptions | 3GPP TS 29.571 [16] | The IMS event reporting options. |
| SessionId | 3GPP TS 29.571 [16] | The IMS session ID. |
| PatchItem | 3GPP TS 29.571 [16] | The patch item. |
| Uri | 3GPP TS 29.571 [16] |  |

6.4.6.2	Structured data types

6.4.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

6.4.6.2.2	Type: ImsEventsSubscription

Table 6.4.6.2.2-1: Definition of type ImsEventsSubscription

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| subscriberId | ImsPublicId | O | 0..1 | Represents the IMS public subscriber identifies that the subscription targeted for. (NOTE) |
| anyUeInd | boolean | O | 0..1 | Identifies whether the request applies to any UE. This attribute is set to "true" if applicable for any UE, otherwise, it is set to "false". (NOTE) |
| SubscriptionId | string | C | 0..1 | Subscription ID. This parameter shall be supplied by the IMS AS in HTTP responses that include an object of ImsEventsSubscription. |
| eventList | map(ImsEventConfiguration) | M | 1..N | A map (list of key-value pairs where referenceId converted from integer to string serves as key. Describes the IMS events to be subscribed in a subscription request or the events successfully subscribed for this subscription in a subscription response. |
| imsReportingOptions | ImsReportingOptions | O | 0..1 | This IE may be included if the NF service consumer wants to describe how the reports of the event to be generated, If this attribute is absent, the default report mode shall be "ON_EVENT_DETECTION". |
| notificationURI | Uri | C | 0..1 | Identifies the recipient of Notifications sent by the NF consumer. This parameter shall be supplied by the NF service consumer in the HTTP POST requests that create the subscriptions for event notifications and in the HTTP PUT requests that update the subscriptions for event notifications. When an NF Service Consumer subscribes on behalf of another NF, the notificationURI identifies a resource under the authority of the other NF. |
| supportedFeatures | SupportedFeatures | O | 0..1 | These are the features supported by the sender of the message. |
| notificationCorrelationId | string | C | 0..1 | This attribute identifies the notification correlation ID shall be present by NF consumer in subscription. The value of this IE shall be unique per subscription for a given NF service consumer. |
| NOTE: If the event subscription applies for a specific UE, subscriberId shall be included; otherwise anyUeInd set to true shall be included. |  |  |  |  |

6.4.6.2.3	Void

6.4.6.2.4	Type: ImsEventsNotification

Table 6.4.6.2.4-1: Definition of type ImsEventsNotification

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| eventNotifications | array(EventNotification) | M | 1..N | Notifications about Individual Events. |
| notificationCorrelationId | string | M | 1 | String identifying a notification correlation Id received in the subscribe . |

6.4.6.2.5	Type: EventNotification

Table 6.4.6.2.5-1: Definition of type EventNotification

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| event | ImsEvent | M | 1 | Event that is notified. |
| eventReport | imsEventReportInfo | O | 0..1 | Report Information per event type |
| subscriberId | ImsPublicId | M | 1 | Represents the IMS public subscriber. |
| timestamp | DateTime | M | 1 | Time at which the event is observed. |
| sessionId | SessionId | O | 0..1 | Identifies the session ID. |

6.4.6.2.6	Void

6.4.6.2.7	Void

6.4.6.2.8	Void

6.4.6.2.9	Void

6.4.6.2.10	Type: ImsEventsSubscriptionCreated

Table 6.4.6.2.10-1: Definition of type ImsEventsSubscriptionCreated

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| imsEventsSubscription | ImsEventsSubscription | M | 1 | This IE shall contain the representation of the created IMS event subscription. |
| eventReports | array(ImsEventReport) | O | 1..N | If immediate report is requested, this IE shall contain the associated status to the event(s) being requested if the status associated to at least one of the requested event(s) is available at the time of subscription. |

6.4.6.2.11	Void

6.4.6.3	Simple data types and enumerations

6.4.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

6.4.6.3.2	Simple data types

The simple data types defined in table 6.4.6.3.2-1 shall be supported.

Table 6.4.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

6.4.6.4	Data types describing alternative data types or combinations of data types

None in this release of the specification.

6.4.6.5	Binary data

None in this release of the specification.

6.4.7	Error Handling

6.4.7.1	General

For the Nimsas_ImsEE API, HTTP error responses shall be supported as specified in clause 4.8 of 3GPP TS 29.501 [5]. Protocol errors and application errors specified in table 5.2.7.2-1 of 3GPP TS 29.500 [4] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.2.7.1-1 of 3GPP TS 29.500 [4].

In addition, the requirements in the following clauses are applicable for the Nimsas_ImsEE API.

6.4.7.2	Protocol Errors

No specific procedures for the Nimsas_ImsEE service are specified.

6.4.7.3	Application Errors

The application errors defined for the Nimsas_ImsEE service are listed in Table 6.1.7.3-1.The common application errors defined in the Table 5.2.7.2-1 in 3GPP TS 29.500 [4] may also be used for the Nimsas_imsEE service. The following application errors listed in Table 6.4.7.3-1 are specific for the Nimsas_imsEE service.

Table 6.4.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| SUBSCRIPTION_NOT_FOUND | 404 Not Found | The service user is not served by this IMS AS. |
| OPERATION_NOT_ALLOWED | 403 Forbidden | Indicates the IMS Event subscription is not allowed to be created/modified or the information of the IMS Event subscription is not allowed to be retrived. |

6.4.8	Feature negotiation

The optional features in table 6.4.8-1 are defined for the Nimsas_ImsEE API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 6.4.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

6.4.9	Security

As indicated in 3GPP TS 33.501 [8] and 3GPP TS 29.500 [4], the access to the Nimsas_ImsEE API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [9]), based on local configuration, using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [10]) plays the role of the authorization server.

If OAuth2 is used, an NF Service Consumer, prior to consuming services offered by the Nimsas_ImsEE API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [10], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Nimsas_EE service.

The Nimsas_ImsEE API defines a single scope "nimsas-ee" for the entire service, and it does not define any additional scopes at resource or operation level.

6.4.10	HTTP redirection

An HTTP request may be redirected to a different IMS AS service instance when using direct or indirect communications (see 3GPP TS 29.500 [4]).

An SCP that reselects a different NF provider (e.g. IMS AS) instance will return the NF Instance ID of the new NF provider instance in the 3gpp-Sbi-Producer-Id header, as specified in clause 6.10.3.4 of 3GPP TS 29.500 [4].

If an NF provider (e.g. IMS AS) redirects a service request to a different NF provider using a 307 Temporary Redirect or 308 Permanent Redirect status code, the identity of the new NF provider towards which the service request is redirected shall be indicated in the 3gpp-Sbi-Target-Nf-Id header of the 307 Temporary Redirect or 308 Permanent Redirect response as specified in clause 6.10.9.1 of 3GPP TS 29.500 [4].

6.5	Nimsas_ImsParameterProvision Service API

6.5.1	API URI

The Nimsas_ImsParameterProvision service shall use the Nimsas_ImsParameterProvision API.

The API URI of the Nimsas_ImsParameterProvision API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests from the NF service consumer towards the NF service producer shall have the Resource URI structure defined in clause 4.4.1 of 3GPP TS 29.501 [5], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [5].

-	The <apiName> shall be "nimsas-ipp".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 6.5.3.

6.5.2	Usage of HTTP

6.5.2.1	General

HTTP/2, IETF RFC 7540 [11], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

The OpenAPI [6] specification of HTTP messages and content bodies for the Nimsas_ImsParameterProvision API is contained in Annex A.

6.5.2.2	HTTP standard headers

6.5.2.2.1	General

See clause 5.2.2 of 3GPP TS 29.500 [4] for the usage of HTTP standard headers.

6.5.2.2.2	Content type

JSON, IETF RFC 8259 [12], shall be used as content type of the HTTP bodies specified in the present specification as specified in clause 5.4 of 3GPP TS 29.500 [4]. The use of the JSON format shall be signalled by the content type "application/json".

"Problem Details" JSON object shall be used to indicate additional details of the error in a HTTP response body and shall be signalled by the content type "application/problem+json", as defined in IETF RFC 7807 [13].

6.5.2.3	HTTP custom headers

The mandatory HTTP custom header fields specified in clause 5.2.3.2 of 3GPP TS 29.500 [4] shall be supported, and the optional HTTP custom header fields specified in clause 5.2.3.3 of 3GPP TS 29.500 [4] may be supported.

6.5.3	Resources

6.5.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.5.3.1-1 depicts the resource URIs structure for the Nimsas_ImsParameterProvision service API.

Figure 6.5.3.1-1: Resource URI structure of the Nimsas_ImsParameterProvision API

Table 6.5.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.5.3.1-1: Resources and methods overview

| Resource purpose/name | Resource URI (relative path after API URI) | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| ImsParameterProvisioningDataEntry | /{imsUeId}/ims-pp-data-store/{afInstanceId} | PUT | Create or update a parameter provisioning data entry per AF for a specific IMS user. |
|  |  | DELETE | Delete a parameter provisioning data entry per AF for a specific IMS user. |

6.5.3.2	Resource: ImsParameterProvisioningDataEntry

6.5.3.2.1	Description

The ImsParameterProvisioningDataEntry resource is used to allow the storage of a collection of parameter provisioning entries for different Application Functions.

In this release, this resource is used to store the IMS user related information created by NF service consumers of Nimsas_ImsParameterProvision service for a given IMS user.

The resource is modelled as Document resource archetype (see clause C.2 of 3GPP TS 29.501 [5]).

6.5.3.2.2	Resource Definition

Resource URI: {apiRoot}/nimsas-ipp/<apiVersion>/{imsUeId}/ims-pp-data-store/{afInstanceId}

This resource shall support the resource URI variables defined in table 6.5.3.2.2-1.

Table 6.5.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5.1 |
| imsUeId | string | Represents the IMS Public Identity (i.e. IMPU). Pattern: "^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦impu-tel\:\+[0-9]{5,15})$" |
| afInstanceId | string | The string identifying the AF instance. |

6.5.3.2.3	Resource Standard Methods

6.5.3.2.3.1	PUT

This method shall support the URI query parameters specified in table 6.5.3.2.3.1-1.

Table 6.5.3.2.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.5.3.2.3.1-2 and the response data structures and response codes specified in table 6.5.3.2.3.1-3.

Table 6.5.3.2.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ImsPpDataEntry | M | 1 | Creates or updates an ImsParameterProvisioningDataEntry resource. |

Table 6.5.3.2.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ImsPpDataEntry | M | 1 | 200 OK | Successful resource update, a response body containing a representation of the modified ImsParameterProvisioningDataEntry resource is returned. |
| ImsPpDataEntry | M | 1 | 201 Created | Successful resource creation, a response body containing a representation of the created ImsParameterProvisioningDataEntry resource is returned. |
| n/a |  |  | 204 No Content | Successful resource update. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the PUT method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.5.3.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative IMS AS (service) instance. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target IMS AS instance ID towards which the request is redirected. |

Table 6.5.3.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative IMS AS (service) instance. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target IMS AS instance ID towards which the request is redirected. |

Table 6.5.3.2.3.1-6: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/nimsas-ipp/<apiVersion>/{imsUeId}/ims-pp-data-store/{afInstanceId} |

6.5.3.2.3.2	DELETE

This method shall support the URI query parameters specified in table 6.5.3.2.3.2-1.

Table 6.5.3.2.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.5.3.2.3.2-2 and the response data structures and response codes specified in table 6.5.3.2.3.2-3.

Table 6.5.3.2.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.5.3.2.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No content | Successful resource deletion. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| Problem Details | O | 0..1 | 404 Not Found | Indicates that the deletion of an ImsParameterProvisioningDataEntry resource failed due to application error. The "cause" attribute may be used to indicate one of the following application errors: - RESOURCE_URI_STRUCTURE_NOT_FOUND |
| NOTE 1: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.5.3.2.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative IMS AS (service) instance. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target IMS AS instance ID towards which the request is redirected. |

Table 6.5.3.2.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative IMS AS (service) instance. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target IMS AS instance ID towards which the request is redirected. |

6.5.3.2.4	Resource Custom Operations

None.

6.5.4	Custom Operations without associated resources

None in this release of specification.

6.5.5	Notifications

None in this release of specification.

6.5.6	Data Model

6.5.6.1	General

This clause specifies the application data model supported by the API.

Table 6.5.6.1-1 specifies the data types defined for the Nimsas_ImsParameterProvision service based interface protocol.

Table 6.5.6.1-1: Nimsas_ImsParameterProvision specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| ImsPpDataEntry | 6.5.6.2.2 | The IMS parameter provision data entry. |

Table 6.5.6.1-2 specifies data types re-used by the Nimsas_ImsParameterProvision service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nimsas_ImsParameterProvision service based interface.

Table 6.5.6.1-2: Nimsas_ImsParameterProvision re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| RcdProperties | 3GPP TS 29.571 [16] | Contains the RCD properties (i.e. RCD information, RCD server address or RCD URL). |

6.5.6.2	Structured data types

6.5.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

6.5.6.2.2	Type: ImsPpDataEntry

Table 6.5.6.2.2-1: Definition of type ImsPpDataEntry

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| rcdProp | RcdProperties | O | 0..1 | Contains the RCD properties (i.e. RCD information, RCD server address or RCD URL). |

6.5.6.3	Simple data types and enumerations

6.5.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

6.5.6.3.2	Simple data types

The simple data types defined in table 6.5.6.3.2-1 shall be supported.

Table 6.5.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

6.5.6.4	Data types describing alternative data types or combinations of data types

None in this release of the specification.

6.5.6.5	Binary data

None in this release of the specification.

6.5.7	Error Handling

6.5.7.1	General

For the Nimsas_ImsParameterProvision API, HTTP error responses shall be supported as specified in clause 4.8 of 3GPP TS 29.501 [5]. Protocol errors and application errors specified in table 5.2.7.2-1 of 3GPP TS 29.500 [4] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.2.7.1-1 of 3GPP TS 29.500 [4].

In addition, the requirements in the following clauses are applicable for the Nimsas_ImsParameterProvision API.

6.5.7.2	Protocol Errors

No specific procedures for the Nimsas_ImsParameterProvision service are specified.

6.5.7.3	Application Errors

The application errors defined for the Nimsas_ImsParameterProvision service are listed in Table 6.5.7.3-1.

Table 6.5.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |

6.5.8	Feature negotiation

The optional features in table 6.5.8-1 are defined for the Nimsas_ImsParameterProvision API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 6.5.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

6.5.9	Security

As indicated in 3GPP TS 33.501 [8] and 3GPP TS 29.500 [4], the access to the Nimsas_ImsParameterProvision API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [9]), based on local configuration, using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [10]) plays the role of the authorization server.

If OAuth2 is used, an NF Service Consumer, prior to consuming services offered by the Nimsas_ImsParameterProvision API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [10], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Nimsas_ImsParameterProvision service.

The Nimsas_ImsParameterProvision API defines a single scope "nimsas-ipp" for the entire service, and it does not define any additional scopes at resource or operation level.

6.5.10	HTTP redirection

An HTTP request may be redirected to a different IMS AS service instance when using direct or indirect communications (see 3GPP TS 29.500 [4]).

An SCP that reselects a different NF provider (e.g. IMS AS) instance will return the NF Instance ID of the new NF provider instance in the 3gpp-Sbi-Producer-Id header, as specified in clause 6.10.3.4 of 3GPP TS 29.500 [4].

If an NF provider (e.g. IMS AS) redirects a service request to a different NF provider using a 307 Temporary Redirect or 308 Permanent Redirect status code, the identity of the new NF provider towards which the service request is redirected shall be indicated in the 3gpp-Sbi-Target-Nf-Id header of the 307 Temporary Redirect or 308 Permanent Redirect response as specified in clause 6.10.9.1 of 3GPP TS 29.500 [4].

Annex A (normative):
OpenAPI specification

A.1	General

This Annex specifies the formal definition of the API(s) defined in the present specification. It consists of OpenAPI specifications in YAML format.

This Annex takes precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API(s).

NOTE 1:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification files contained in this 3GPP Technical Specification are available on a Git-based repository that uses the GitLab software version control system (see clause 5.3.1 of 3GPP TS 29.501 [5] and clause 5B of 3GPP TR 21.900 [7]).

A.2	Nimsas_SessionEventControl API

openapi: 3.0.0

info:

title: 'IMS AS SessionEventControl Service'

version: 1.1.0-alpha.4

description: |

Nimsas_SessionEventControl Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.175 V19.3.0; IP Multimedia Subsystem; IP Multimedia Subsystem (IMS) Application

Server (AS) Services; Stage 3

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.175/'

servers:

- url: '{apiRoot}/nimsas-sec/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- nimsas-sec

paths:

/session-event-subscriptions:

post:

# This is a pseudo operation, NF service consumers shall NOT invoke this method!

summary: Nimsas_SessionEventControl Subscribe service operation, pseudo operation

tags:

- Session Event Subscriptions (Collection)

operationId: Subscribe

requestBody:

required: true

content:

application/json:

# Unspecified schema for the JSON body, since this is not used by consumer or producer.

schema: {}

responses:

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

eeNotification:

'{SessionEventNotificationUri}':

# The notification URI is provided by local configuration or via NRF.

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SessionEventNotification'

responses:

'204':

description: No Content, notification was succesfull.

'307':

description: Temporary Redirect

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RedirectResponse'

headers:

Location:

description: >

The URI pointing to the resource located on the redirect target NF

service consumer.

required: true

schema:

type: string

'308':

description: Permanent Redirect

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RedirectResponse'

headers:

Location:

description: >

The URI pointing to the resource located on the redirect target NF

service consumer.

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nimsas-sec: Access to the Nimsas_SessionEventControl API

schemas:

# STRUCTURED DATA TYPES

SessionEventNotification:

description: The session event notification item.

type: object

required:

- notificationEvent

- sessionId

properties:

notificationEvent:

$ref: '#/components/schemas/NotificationEvent'

sessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SessionId'

sessionInfo:

$ref: '#/components/schemas/SessionInfo'

mediaInfoList:

description: >

Contains a media related information. The mediaId attribute within the MediaInfo

data type is the key of the map.

type: object

additionalProperties:

$ref: '#/components/schemas/MediaInfo'

minItems: 1

psDataOff:

$ref: '#/components/schemas/PsDataOffStatus'

mediaInfoSet:

type: array

minItems: 1

items:

$ref: 'TS29175_Nimsas_ImsSessionManagement.yaml#/components/schemas/MediaInfoExternal'

NotificationEvent:

description: The notification event information.

type: object

required:

- eventType

properties:

eventType:

$ref: '#/components/schemas/EventType'

eventInitiator:

$ref: '#/components/schemas/EventInitiator'

SessionInfo:

description: The session information related to the notification event.

type: object

properties:

callingIdentity:

$ref: 'TS29562_Nhss_imsSDM.yaml#/components/schemas/ImsPublicId'

calledIdentity:

$ref: 'TS29562_Nhss_imsSDM.yaml#/components/schemas/ImsPublicId'

sessionCase:

$ref: '#/components/schemas/SessionCase'

MediaInfo:

description: The media information related to the notification event.

type: object

required:

- mediaId

- mediaType

properties:

mediaId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MediaId'

mediaType:

$ref: '#/components/schemas/MediaType'

dcMediaSpec:

$ref: '#/components/schemas/DcMediaSpec'

mediaSuspended:

type: boolean

mediaRemovalInd:

type: boolean

enum:

- true

DcMediaSpec:

description: >

The data channel media specification information contains media description attributes.

type: object

required:

- streams

- receivedDcEndpoint

properties:

streams:

description: >

Contains a data channel mapping and configuration information. The streamId

attribute within the DcStream data type is the key of the map.

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DcEndpoint'

minItems: 1

maxMessageSize:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MaxMessageSize'

receivedDcEndpoint:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DcEndpoint'

bdcUsedBy:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BdcUsedBy'

appBindingInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AppBindingInfo'

qosHintInfo:

$ref: '#/components/schemas/QosHintInfo'

QosHintInfo:

description: The QoS hint information used for the media.

type: object

properties:

loss:

description: the maximum desirable end-to-end transport level packet loss rate

type: number

format: zero-based-integer or non-zero-real

localLoss:

description: the maximum desirable end-to-end transport level packet loss rate applied across the local link

type: number

format: zero-based-integer or non-zero-real

latency:

description: the maximum desirable end-to-end transport level packet latency

type: number

format: zero-based-integer or non-zero-real

localLatency:

description: the maximum desirable end-to-end transport level packet latency applied across the local link

type: number

format: zero-based-integer or non-zero-real

# ENUMS

EventType:

description: The type of the event for which the notification is generated.

anyOf:

- type: string

enum:

- SESSION_ESTABLISHMENT_REQUEST

- SESSION_ESTABLISHMENT_PROGRESS

- SESSION_ESTABLISHMENT_ALERTING

- SESSION_ESTABLISHMENT_SUCCESS

- SESSION_ESTABLISHMENT_FAILURE

- SESSION_ESTABLISHMENT_CANCEL

- MEDIA_CHANGE_REQUEST

- MEDIA_CHANGE_SUCCESS

- MEDIA_CHANGE_FAILURE

- MEDIA_CHANGE_CANCEL

- SESSION_TERMINATION

- EXTERNAL_SESSION_UPDATE

- EXTERNAL_SESSION_CREATE

- PS_DATA_OFF_STATUS_CHANGE

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

MediaType:

description: The type of the media for which the notification is generated.

anyOf:

- type: string

enum:

- DC

- AUDIO

- VIDEO

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

SessionCase:

description: The type of the session case.

anyOf:

- type: string

enum:

- ORIGINATING_IMS_SESSION

- TERMINATING_IMS_SESSION

- EXTERNAL_INITIATED_IMS_SESSION

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

EventInitiator:

description: The initiator of the event for which the notification is generated.

anyOf:

- type: string

enum:

- SERVED_IMS_SUBSCRIBER

- REMOTE_IMS_SUBSCRIBER

- EXTERNAL_PARTY

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

PsDataOffStatus:

description: The active/inactive status of PS Data Off.

anyOf:

- type: string

enum:

- ACTIVE

- INACTIVE

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

A.3	Nimsas_MediaControl API

openapi: 3.0.0

info:

title: 'IMS AS MediaControl Service'

version: 1.1.1-alpha.3

description: |

Nimsas_MediaControl Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.175 V19.3.0; IP Multimedia Subsystem; IP Multimedia Subsystem (IMS) Application

Server (AS) Services; Stage 3

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.175'

servers:

- url: '{apiRoot}/nimsas-mc/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- nimsas-mc

paths:

/call-sessions/{sessionId}/media-instruction:

post:

summary: Update

operationId: UpdateCallSession

tags:

- Individual call session (Document)

parameters:

- name: sessionId

in: path

description: Session ID assigned by the IMS AS during the IMS session setup.

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SessionId'

requestBody:

description: Representation of the updates to apply to the call session.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MediaInstructionData'

responses:

'200':

description: >

Successful update of the call session, when the IMS AS needs to return information

in the response.

content:

application/json:

schema:

$ref: '#/components/schemas/MediaInstructionData'

'204':

description: >

Successful update of the call session, when the IMS AS does not need to return

information in the response.

'307':

description: Temporary Redirect

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RedirectResponse'

headers:

Location:

description: The URI pointing to the resource located on the redirect target IMS AS.

schema:

type: string

'308':

description: Permanent Redirect

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RedirectResponse'

headers:

Location:

description: The URI pointing to the resource located on the redirect target IMS AS.

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'501':

$ref: 'TS29571_CommonData.yaml#/components/responses/501'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nimsas-mc: Access to the Nimsas_MediaControl API.

schemas:

# STRUCTURED DATA TYPES

MediaInstructionData:

description: The media instruction data for a specific session.

type: object

required:

- sessionId

- mediaInstructionSet

properties:

sessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SessionId'

mediaInstructionSet:

description: >

Contains a set of instructions for each media flow to control. The mediaId

attribute within the MediaInstructions data type is the key of the map.

type: object

additionalProperties:

$ref: '#/components/schemas/MediaInstructions'

minProperties: 1

MediaInstructions:

description: The media instructions for a specific media flow.

type: object

required:

- mediaId

- mediaResourceType

properties:

mediaId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MediaId'

associatedMediaId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MediaId'

mediaResourceType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MediaResourceType'

mediaInstruction:

$ref: '#/components/schemas/MediaInstruction'

dcMediaSpecification:

$ref: '#/components/schemas/DcMediaSpecification'

arMediaSpecification:

$ref: '#/components/schemas/ArMediaSpecification'

mediaProcessingUrl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

audioVideoReNegotiationInd:

$ref: '#/components/schemas/AudioVideoReNegotiationInd'

avatarMedia:

$ref: 'TS29176_Nmf_MRM.yaml#/components/schemas/AvatarMedia'

DcMediaSpecification:

description: The data channel media specification information.

type: object

required:

- streams

- mediaProxyConfig

properties:

mediaProxyConfig:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MediaProxy'

replaceHttpUrls:

description: >

Contains a list of replacement HTTP URLs. The streamId

attribute within the ReplaceHttpUrl data type is the key of the map.

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ReplaceHttpUrl'

minProperties: 1

mdc1EndpointDcsf:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MdcEndpoint'

mdc1EndpointMf:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MdcEndpoint'

mdc2EndpointInfo:

$ref: '#/components/schemas/Mdc2EndpointInfo'

streams:

description: >

Contains a data channel mapping and configuration information. The streamId

attribute within the DcStream data type is the key of the map.

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DcStream'

minProperties: 1

bdcUsedBy:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BdcUsedBy'

appBindingInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AppBindingInfo'

qosHintInfo:

$ref: 'TS29175_Nimsas_SessionEventControl.yaml#/components/schemas/QosHintInfo'

interworkingInfo:

$ref: '#/components/schemas/InterworkingInstruction'

dcIwkInd:

type: boolean

default: false

ArMediaSpecification:

description: The AR media specification information.

type: object

required:

- mediaProcessingSpec

properties:

mediaProcessingSpec:

type: string

Mdc2EndpointInfo:

description: Represents the negotiated MDC2 media endpoint information.

type: object

properties:

mdc2Protocol:

$ref: 'TS29176_Nmf_MRM.yaml#/components/schemas/Mdc2Protocol'

mdc2EndpointDcAs:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MdcEndpoint'

mdc2EndpointMf:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MdcEndpoint'

AudioVideoReNegotiationInd:

description: The audio/video media re-negotiation indication information.

type: object

required:

- reNegotiation

properties:

reNegotiation:

type: boolean

mediaConnSide:

$ref: '#/components/schemas/MediaConnSide'

mediaDirection:

$ref: '#/components/schemas/SendRecvMode'

InterworkingInstruction:

description: The interworking instruction of data channel transcoding.

type: object

required:

- transcodeMode

properties:

transcodeMode:

$ref: '#/components/schemas/TranscodeMode'

addTransInfo:

type: string

# ENUMS

MediaInstruction:

description: The media instruction for handling a specific media flow.

anyOf:

- type: string

enum:

- TERMINATE_MEDIA

- ORIGINATE_MEDIA

- TERMINATE_AND_ORIGINATE_MEDIA

- UPDATE_MEDIA

- DELETE_MEDIA

- REJECT_MEDIA

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

MediaConnSide:

description: The audio or video media connection side.

anyOf:

- type: string

enum:

- BOTH

- TERMINATING_SIDE

- ORIGINATING_SIDE

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

SendRecvMode:

description: The send and receive mode of media stream.

anyOf:

- type: string

enum:

- SENDRECV

- SENDONLY

- RECVONLY

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

TranscodeMode:

description: The mode of the trancoding.

anyOf:

- type: string

enum:

- DC_TO_VIDEO

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

A.4	Nimsas_ImsSessionManagement API

openapi: 3.0.0

info:
  title: 'IMS AS ImsSessionManagement Service'
  version: 1.0.0-alpha.2
  description: |
    Nimsas_ImsSessionManagement Service.  
    © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).  
    All rights reserved.

externalDocs:
  description: >
    3GPP TS 29.175 V19.3.0; IP Multimedia Subsystem; IP Multimedia Subsystem (IMS) Application
    Server (AS) Services; Stage 3
  url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.175'

servers:
  - url: '{apiRoot}/nimsas-ism/v1'
    variables:
      apiRoot:
        default: https://example.com
        description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:
  - {}
  - oAuth2ClientCredentials:
      - nimsas-ism

paths:
  /ims-sessions:
    post:
      summary: Create IMS Session
      operationId: ImsSessionCreate
      tags:
        - IMS Sessions (Collection)
      requestBody:
        description: Creates a new Individual IMS Session resource.
        required: true
        content:
          application/json:
            schema:
              $ref: '#/components/schemas/ImsSessionInfo'
      responses:
        '201':
          description: >
            The creation of an Individual IMS Session resource is confirmed and
            a representation of that resource is returned.
          content:
            application/json:
              schema:
                $ref: '#/components/schemas/ImsSessionInfo'
          headers:
            Location:
              description: > 
                The URI of the newly created resource, according to the structure:
                {apiRoot}/nimsas-ism/<apiVersion>/ims-sessions/{sessionId}.
              schema:
                type: string
        '307':
          description: Temporary Redirect
          content:
            application/json:
              schema:
                $ref: 'TS29571_CommonData.yaml#/components/schemas/RedirectResponse'
          headers:
            Location:
              description: The URI pointing to the resource located on the redirect target IMS AS.
              schema:
                type: string
        '308':
          description: Permanent Redirect
          content:
            application/json:
              schema:
                $ref: 'TS29571_CommonData.yaml#/components/schemas/RedirectResponse'
          headers:
            Location:
              description: The URI pointing to the resource located on the redirect target IMS AS.
              schema:
                type: string
        '400':
          $ref: 'TS29571_CommonData.yaml#/components/responses/400'
        '401':
          $ref: 'TS29571_CommonData.yaml#/components/responses/401'
        '403':
          $ref: 'TS29571_CommonData.yaml#/components/responses/403'
        '404':
          $ref: 'TS29571_CommonData.yaml#/components/responses/404'
        '406':
          $ref: 'TS29571_CommonData.yaml#/components/responses/406'
        '411':
          $ref: 'TS29571_CommonData.yaml#/components/responses/411'
        '413':
          $ref: 'TS29571_CommonData.yaml#/components/responses/413'
        '415':
          $ref: 'TS29571_CommonData.yaml#/components/responses/415'
        '429':
          $ref: 'TS29571_CommonData.yaml#/components/responses/429'
        '500':
          $ref: 'TS29571_CommonData.yaml#/components/responses/500'
        '501':
          $ref: 'TS29571_CommonData.yaml#/components/responses/501'
        '503':
          $ref: 'TS29571_CommonData.yaml#/components/responses/503'
        default:
          $ref: 'TS29571_CommonData.yaml#/components/responses/default'

  /ims-sessions/{sessionId}:
    patch:
      summary: modify the IMS session
      operationId: ImsSessionModify
      tags:
        - IMS Session Modification
      parameters:
        - name: sessionId
          in: path
          description: Identifies an individual IMS session.
          required: true
          schema:
            $ref: 'TS29571_CommonData.yaml#/components/schemas/SessionId'
      requestBody:
        content:
          application/json-patch+json:
            schema:
              type: array
              items:
                $ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'
              minItems: 1
        required: true
      responses:
        '200':
          description: Successful update on the IMS session.
          content:
            application/json:
              schema:
                $ref: '#/components/schemas/ImsSessionInfo'
        '204':
          description: >
            No content, IMS AS accepts the request change which is to 
            delete the existing media.
        '307':
          description: Temporary Redirect
          content:
            application/json:
              schema:
                $ref: 'TS29571_CommonData.yaml#/components/schemas/RedirectResponse'
          headers:
            Location:
              description: The URI pointing to the resource located on the redirect target IMS AS.
              schema:
                type: string
        '308':
          description: Permanent Redirect
          content:
            application/json:
              schema:
                $ref: 'TS29571_CommonData.yaml#/components/schemas/RedirectResponse'
          headers:
            Location:
              description: The URI pointing to the resource located on the redirect target IMS AS.
              schema:
                type: string
        '400':
          $ref: 'TS29571_CommonData.yaml#/components/responses/400'
        '401':
          $ref: 'TS29571_CommonData.yaml#/components/responses/401'
        '403':
          $ref: 'TS29571_CommonData.yaml#/components/responses/403'
        '404':
          $ref: 'TS29571_CommonData.yaml#/components/responses/404'
        '411':
          $ref: 'TS29571_CommonData.yaml#/components/responses/411'
        '413':
          $ref: 'TS29571_CommonData.yaml#/components/responses/413'
        '415':
          $ref: 'TS29571_CommonData.yaml#/components/responses/415'
        '429':
          $ref: 'TS29571_CommonData.yaml#/components/responses/429'
        '500':
          $ref: 'TS29571_CommonData.yaml#/components/responses/500'
        '502':
          $ref: 'TS29571_CommonData.yaml#/components/responses/502'
        '503':
          $ref: 'TS29571_CommonData.yaml#/components/responses/503'
        default:
          $ref: 'TS29571_CommonData.yaml#/components/responses/default'

put:

summary: Update an existing Individual IMS Session resource.

operationId: ImsSessionUpdate

tags:

- Individual IMS Session (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ImsSessionInfo'

responses:

'200':

description: >

OK. The Individual IMS Session resource is successfully updated and a representation

of the updated resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/ImsSessionInfo'

'204':

description: >

No Content. The Individual IMS Session Management resource is

successfully updated and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'409':

$ref: 'TS29122_CommonData.yaml#/components/responses/409'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

    delete:
      summary: delete the IMS session
      operationId: ImsSessionDelete
      tags:
        - IMS Session Deletion
      parameters:
        - name: sessionId
          in: path
          description: Identifies an individual IMS session.
          required: true
          schema:
            $ref: 'TS29571_CommonData.yaml#/components/schemas/SessionId'
      responses:
        '204':
          description: No content, successful delete of the resource identified by sessionId.
        '307':
          description: Temporary Redirect
          content:
            application/json:
              schema:
                $ref: 'TS29571_CommonData.yaml#/components/schemas/RedirectResponse'
          headers:
            Location:
              description: The URI pointing to the resource located on the redirect target IMS AS.
              schema:
                type: string
        '308':
          description: Permanent Redirect
          content:
            application/json:
              schema:
                $ref: 'TS29571_CommonData.yaml#/components/schemas/RedirectResponse'
          headers:
            Location:
              description: The URI pointing to the resource located on the redirect target IMS AS.
              schema:
                type: string
        '400':
          $ref: 'TS29571_CommonData.yaml#/components/responses/400'
        '401':
          $ref: 'TS29571_CommonData.yaml#/components/responses/401'
        '403':
          $ref: 'TS29571_CommonData.yaml#/components/responses/403'
        '404':
          $ref: 'TS29571_CommonData.yaml#/components/responses/404'
        '429':
          $ref: 'TS29571_CommonData.yaml#/components/responses/429'
        '500':
          $ref: 'TS29571_CommonData.yaml#/components/responses/500'
        '502':
          $ref: 'TS29571_CommonData.yaml#/components/responses/502'
        '503':
          $ref: 'TS29571_CommonData.yaml#/components/responses/503'
        default:
          $ref: 'TS29571_CommonData.yaml#/components/responses/default'

  /ims-sessions/event-subscriptions:
    post:
    # This is a pseudo operation, NF service consumers shall NOT invoke this method!
      summary: Nimsas_ImsSessionManagement Subscribe service operation, pseudo operation
      tags:
        - IMS Session Event Subscriptions (Collection)
      operationId: Subscribe
      requestBody:
        required: true
        content:
          application/json:
            # Unspecified schema for the JSON body, since this is not used by consumer or producer.
            schema: {}
      responses:
        default:
          $ref: 'TS29571_CommonData.yaml#/components/responses/default'

      callbacks:
        eventNotification:
          '{$request.body#/notifUri}':
          # The notification URI of the Service Consumer to receive the IMS session events.
            post:
              requestBody:
                required: true
                content:
                  application/json:
                    schema:
                      $ref: '#/components/schemas/ImsSessionEventNotification'
              responses:
                '204':
                  description: No Content, notification was succesfull.
                '307':
                  description: Temporary Redirect
                  content:
                    application/json:
                      schema:
                        $ref: 'TS29571_CommonData.yaml#/components/schemas/RedirectResponse'
                  headers:
                    Location:
                      description: >
                        The URI pointing to the resource located on the redirect target NF
                        service consumer.
                      required: true
                      schema:
                        type: string
                '308':
                  description: Permanent Redirect
                  content:
                    application/json:
                      schema:
                        $ref: 'TS29571_CommonData.yaml#/components/schemas/RedirectResponse'
                  headers:
                    Location:
                      description: >
                        The URI pointing to the resource located on the redirect target NF
                        service consumer.
                      required: true
                      schema:
                        type: string
                '400':
                  $ref: 'TS29571_CommonData.yaml#/components/responses/400'
                '401':
                  $ref: 'TS29571_CommonData.yaml#/components/responses/401'
                '403':
                  $ref: 'TS29571_CommonData.yaml#/components/responses/403'
                '404':
                  $ref: 'TS29571_CommonData.yaml#/components/responses/404'
                '411':
                  $ref: 'TS29571_CommonData.yaml#/components/responses/411'
                '413':
                  $ref: 'TS29571_CommonData.yaml#/components/responses/413'
                '415':
                  $ref: 'TS29571_CommonData.yaml#/components/responses/415'
                '429':
                  $ref: 'TS29571_CommonData.yaml#/components/responses/429'
                '500':
                  $ref: 'TS29571_CommonData.yaml#/components/responses/500'
                '502':
                  $ref: 'TS29571_CommonData.yaml#/components/responses/502'
                '503':
                  $ref: 'TS29571_CommonData.yaml#/components/responses/503'
                default:
                  $ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

  securitySchemes:
    oAuth2ClientCredentials:
      type: oauth2
      flows:
        clientCredentials:
          tokenUrl: '{nrfApiRoot}/oauth2/token'
          scopes:
            nimsas-ism: Access to the Nimsas_ImsSessionManagement API.

  schemas:

# STRUCTURED DATA TYPES

    ImsSessionInfo:
      description: The IMS session information and the media list used in the IMS session.
      type: object
      required:
        - sessionInfo
        - mediaInfoSet
      properties:
        sessionInfo:
          $ref: '#/components/schemas/SessionInfoSM'
        mediaInfoSet:
          description: Includes a list of media related information.
          type: array
          items:
            $ref: '#/components/schemas/MediaInfoExternal'
          minItems: 1
        sessionId:
          $ref: 'TS29571_CommonData.yaml#/components/schemas/SessionId'
        notifUri:
          $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'
        notifCorrelationId:
          type: string

    SessionInfoSM:
      description: The session information to be used.
      type: object
      required:
        - callingId
        - calledId
      properties:
        callingId:
          $ref: 'TS29562_Nhss_imsSDM.yaml#/components/schemas/ImsPublicId'
        calledId:
          $ref: 'TS29562_Nhss_imsSDM.yaml#/components/schemas/ImsPublicId'

    MediaInfoExternal:
      description: The media information used in the external created or updated IMS session.
      type: object
      required:
        - mediaType
        - mediaCorrectionId
      properties:
        mediaType:
          $ref: 'TS29175_Nimsas_SessionEventControl.yaml#/components/schemas/MediaType'
        mediaCorrectionId:
          type: string
        dcMediaParas:
          description: The information of the created data channel media.
          type: array
          items:
            $ref: '#/components/schemas/DcMediaPara'
          minItems: 1

    DcMediaPara:
      description: The parameters of the IMS DC media.
      type: object
      required:
        - dcType
      properties:
        dcType:
          $ref: '#/components/schemas/DcType'
        adcType:
          $ref: '#/components/schemas/AdcType'
        appBindingInfo:
          $ref: 'TS29571_CommonData.yaml#/components/schemas/AppBindingInfo'
        targetIds:
          description: Representes the public identity(ies) of the IMS Subscriber the DC is targeted.
          type: array
          items:
            $ref: '#/components/schemas/TargetIdInfo'
          minItems: 1
        targetId:
          $ref: 'TS29562_Nhss_imsSDM.yaml#/components/schemas/ImsPublicId'
        mdc2EndpointInfo:
          $ref: 'TS29175_Nimsas_MediaControl.yaml#/components/schemas/Mdc2EndpointInfo'
        
    ImsSessionEventNotification:
      description: The IMS Session event notification.
      type: object
      required:
        - sessionId
        - eventType
        - notifCorrelationId
      properties:
        sessionId:
          $ref: 'TS29571_CommonData.yaml#/components/schemas/SessionId'
        eventType:
          $ref: '#/components/schemas/ImsSessionEventType'
        notifCorrelationId:
          type: string
        mediaInfoSet:
          description: Includes a list of media related information.
          type: array
          items:
            $ref: '#/components/schemas/MediaInfoExternal'
          minItems: 1

    TargetIdInfo:
      description: The public identity(ies) of the IMS Subscriber the DC is targeted.
      type: object
      properties:
        callingIdentity:
          $ref: 'TS29562_Nhss_imsSDM.yaml#/components/schemas/ImsPublicId'
        calledIdentity:
          $ref: 'TS29562_Nhss_imsSDM.yaml#/components/schemas/ImsPublicId'



# ENUMS

    DcType:
      description: The type of data channel.
      anyOf:
        - type: string
          enum:
            - BDC
            - ADC
        - type: string
          description: >
            This string provides forward-compatibility with future extensions to the enumeration
            and is not used to encode content defined in the present version of this API.

    AdcType:
      description: The type of the application data channel.
      anyOf:
        - type: string
          enum:
            - P2P
            - P2A
            - P2A2P
        - type: string
          description: >
            This string provides forward-compatibility with future extensions to the enumeration
            and is not used to encode content defined in the present version of this API.

    ImsSessionEventType:
      description: The IMS session event type for which the notification is generated.
      anyOf:
        - type: string
          enum:
            - SESSION_ESTABLISHMENT_SUCCESS
            - SESSION_ESTABLISHMENT_FAILURE
            - MEDIA_CHANGE_SUCCESS
            - MEDIA_CHANGE_FAILURE
            - SESSION_TERMINATION
        - type: string
          description: >
            This string provides forward-compatibility with future extensions to the enumeration
            and is not used to encode content defined in the present version of this API.

A.5	Nimsas_ImsEE API

openapi: 3.0.0

info:

title: 'IMS AS ImsEventExposure Service'

version: 1.0.0-alpha.2

description: |

Nimsas_ImsEE Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.175 V19.3.0; IP Multimedia Subsystem; IP Multimedia Subsystem (IMS) Application

Server (AS) Services; Stage 3

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.175'

servers:

- url: '{apiRoot}/nimsas-ee/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- nimsas-ee

paths:

/subscriptions:

post:

summary: Subscribe to IMS Event

operationId: ImsEventSubscribe

tags:

- IMS Event Subscriptions (Collection)

requestBody:

description: Creates a new Individual IMS Event Subscription resource.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ImsEventsSubscription'

responses:

'201':

description: >

Successful creation of the Individual IMS Event Subscription resource,

when the IMS AS needs to return information in the response.

content:

application/json:

schema:

$ref: '#/components/schemas/ImsEventsSubscriptionCreated'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure:

{apiRoot}/nimsas-ee/<apiVersion>/subscriptions/{subscriptionId}.

schema:

type: string

'307':

description: Temporary Redirect

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RedirectResponse'

headers:

Location:

description: The URI pointing to the resource located on the redirect target IMS AS.

schema:

type: string

'308':

description: Permanent Redirect

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RedirectResponse'

headers:

Location:

description: The URI pointing to the resource located on the redirect target IMS AS.

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'501':

$ref: 'TS29571_CommonData.yaml#/components/responses/501'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

eventNotification:

'{$request.body#/notificationURI}':

# The notification URI is assigned within the Individual IMS Event Subscription.

post:

requestBody:

required: true

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/ImsEventsNotification'

minItems: 1

responses:

'204':

description: No Content, the receipt of the Notification is acknowledged.

'307':

description: Temporary Redirect

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RedirectResponse'

headers:

Location:

description: >

The URI pointing to the resource located on the redirect target NF

service consumer.

required: true

schema:

type: string

'308':

description: Permanent Redirect

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RedirectResponse'

headers:

Location:

description: >

The URI pointing to the resource located on the redirect target NF

service consumer.

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

get:

summary: Retrieve the subscription information

operationId: GetSubscriptionInfo

tags:

- Subscription information retrieval

parameters:

- name: subscriptionId

in: path

description: Individual IMS event subscription Identity

required: true

schema:

type: string

responses:

'200':

description: >

A response body containing representation of the Individual IMS Event Subcription resource.

content:

application/json:

schema:

$ref: '#/components/schemas/ImsEventsSubscription'

'307':

description: Temporary Redirect

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RedirectResponse'

headers:

Location:

description: The URI pointing to the resource located on the redirect target IMS AS.

schema:

type: string

'308':

description: Permanent Redirect

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RedirectResponse'

headers:

Location:

description: The URI pointing to the resource located on the redirect target IMS AS.

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

put:

summary: Update IMS Event Subscription

operationId: ImsEventSubsModify

tags:

- IMS Event Subscription Modification

parameters:

- name: subscriptionId

in: path

description: Individual IMS event subscription Identity

required: true

schema:

type: string

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/ImsEventsSubscription'

required: true

responses:

'200':

description: >

Successful update of the subscription, when the IMS AS needs

to return information in the response.

content:

application/json:

schema:

$ref: '#/components/schemas/ImsEventsSubscriptionCreated'

'204':

description: No content

'307':

description: Temporary Redirect

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RedirectResponse'

headers:

Location:

description: The URI pointing to the resource located on the redirect target IMS AS.

schema:

type: string

'308':

description: Permanent Redirect

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RedirectResponse'

headers:

Location:

description: The URI pointing to the resource located on the redirect target IMS AS.

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'409':

$ref: 'TS29571_CommonData.yaml#/components/responses/409'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

patch:

summary: Partial update IMS Event Subscription

operationId: ImsEventSubsPartialModify

tags:

- IMS Event Subscription Partial Modification

parameters:

- name: subscriptionId

in: path

description: Individual IMS event subscription Identity

required: true

schema:

type: string

requestBody:

content:

application/json-patch+json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

minItems: 1

required: true

responses:

'200':

description: >

Successful update of the subscription, when the IMS AS needs

to return information in the response.

content:

application/json:

schema:

$ref: '#/components/schemas/ImsEventsSubscriptionCreated'

'204':

description: No content

'307':

description: Temporary Redirect

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RedirectResponse'

headers:

Location:

description: The URI pointing to the resource located on the redirect target IMS AS.

schema:

type: string

'308':

description: Permanent Redirect

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RedirectResponse'

headers:

Location:

description: The URI pointing to the resource located on the redirect target IMS AS.

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: delete

operationId: ImsEventSubsDelete

tags:

- IMS Event Subscription Deletion

parameters:

- name: subscriptionId

in: path

description: Individual IMS event subscription Identity

required: true

schema:

type: string

responses:

'204':

description: No content

'307':

description: Temporary Redirect

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RedirectResponse'

headers:

Location:

description: The URI pointing to the resource located on the redirect target IMS AS.

schema:

type: string

'308':

description: Permanent Redirect

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RedirectResponse'

headers:

Location:

description: The URI pointing to the resource located on the redirect target IMS AS.

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nimsas-ee: Access to the Nimsas_ImsEE API.

schemas:

# STRUCTURED DATA TYPES

ImsEventsSubscription:

description: Represents an Individual IMS Event Subscription resource.

type: object

required:

- eventList

properties:

eventList:

description: >

the IMS events to be subscribed or successfully subscribed in a

subscription response within the referenceId converted from integer

to string is the key of the map.

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ImsEventConfiguration'

minProperties: 1

subscriberId:

$ref: 'TS29562_Nhss_imsSDM.yaml#/components/schemas/ImsPublicId'

anyUeInd:

type: boolean

subscriptionId:

type: string

imsReportingOptions:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ImsReportingOptions'

notificationURI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

notificationCorrelationId:

type: string

ImsEventsNotification:

description: Notifications about Individual Events.

type: object

required:

- eventNotifications

- notificationCorrelationId

properties:

eventNotifications:

description: Notifications about Individual Events.

type: array

items:

$ref: '#/components/schemas/EventNotification'

minItems: 1

notificationCorrelationId:

type: string

EventNotification:

description: The notification information for a specific event.

type: object

required:

- event

- subscriberId

- timestamp

properties:

event:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ImsEvent'

subscriberId:

$ref: 'TS29562_Nhss_imsSDM.yaml#/components/schemas/ImsPublicId'

timestamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

eventReport:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ImsEventReportInfo'

sessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SessionId'

ImsEventsSubscriptionCreated:

description: >

Represents the subscription of the created/modified IMS event subscriptiona and

the immediate reports

type: object

required:

- imsEventsSubscription

properties:

imsEventsSubscription:

$ref: '#/components/schemas/ImsEventsSubscription'

eventReports:

description: >

Contains the associated status to the event(s) being requested if

the status associated to at least one of the requested event(s)

is available at the time of subscription.

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ImsEventReport'

minItems: 1

# ENUMS

A.6	Nimsas_ImsParameterProvision API

openapi: 3.0.0

info:

title: 'IMS AS ImsParameterProvision Service'

version: 1.0.0-alpha.1

description: |

Nimsas_ImsParameterProvision Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.175 V19.3.0; IP Multimedia Subsystem; IP Multimedia Subsystem (IMS) Application

Server (AS) Services; Stage 3

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.175'

servers:

- url: '{apiRoot}/nimsas-ipp/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- nimsas-ipp

paths:

/{imsUeId}/ims-pp-data-store/{afInstanceId}:

put:

summary: Creates or updates the parameter provisioning data entry.

operationId: CreateUpdatePpDataEntry

tags:

- ImsParameterProvisioningDataEntry

parameters:

- name: imsUeId

in: path

description: Contains the IMS public identity.

required: true

schema:

type: string

pattern: '^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}|impu-tel\:\+[0-9]{5,15})$'

- name: afInstanceId

in: path

description: Contains the AF instance identity.

required: true

schema:

type: string

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/ImsPpDataEntry'

required: true

responses:

'201':

description: >

Created, a representation of the created ImsParameterProvisioningDataEntry resource

is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/ImsPpDataEntry'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure:

{apiRoot}/nhss-ims-uecm/v1/{imsUeId}/imsas-registration

required: true

schema:

type: string

'200':

description: >

OK, a representation of the updated ImsParameterProvisioningDataEntry resource

is returned..

content:

application/json:

schema:

$ref: '#/components/schemas/ImsPpDataEntry'

'204':

description: No content, the ImsParameterProvisioningDataEntry resource is updated.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Delete the parameter provisioning data entry.

operationId: DeletePpDataEntry

tags:

- ImsParameterProvisioningDataEntry

parameters:

- name: imsUeId

in: path

description: Contains the IMS public identity.

required: true

schema:

type: string

pattern: '^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}|impu-tel\:\+[0-9]{5,15})$'

- name: afInstanceId

in: path

description: Contains the AF instance identity.

required: true

schema:

type: string

responses:

'204':

description: No content, the ImsParameterProvisioningDataEntry resource is deleted.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nimsas-ipp: Access to the Nimsas_ImsParameterProvision API

schemas:

# Structured data types

ImsPpDataEntry:

description: Contains the IMS parameter provision data i.e. the RCD properties.

type: object

properties:

rcdProp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RcdProperties'

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2023-04 | CT4#115e | C4-231514 |  |  |  | TS skeleton | 0.0.0 |
| 2023-04 | CT4#115e | C4-231551 |  |  |  | Inclusion of pCRs agreed at CT4#115e, including C4-231214, C4-231217, C4-231224, C4-231225, C4-231514, C4-231515, C4-231516, C4-231517. | 0.1.0 |
| 2023-05 | CT4#116 | C4-232553 |  |  |  | Inclusion of pCRs agreed at CT4#116, including C4-232142, C4-232145, C4-232504, C4-232505. | 0.2.0 |
| 2023-09 | CT4#117 | C4-233810 |  |  |  | Inclusion of pCRs agreed at CT4#117, including C4-233250,C4-233254,C4-233255,C4-233256,C4-233700,C4-233701,C4-233704,C4-233856. | 0.3.0 |
| 2023-10 | CT4#118 | C4-234583 |  |  |  | Inclusion of pCRs agreed at CT4#118, including C4-234172, C4-234174, C4-234175, C4-234177, C4-234545, C4-234547. | 0.4.0 |
| 2023-12 | CT4#119 | C4-235660 |  |  |  | Inclusion of pCRs agreed at CT4#119, including C4-235221, C4-235464, C4-235465, C4-235466, C4-235467, C4-235468, C4-235471 | 0.5.0 |
| 2023-12 | CT#102 | CP-233022 |  |  |  | TS presented for information | 1.0.0 |
| 2024-03 | CT4#121 | C4-240852 |  |  |  | Inclusion of pCRs agreed at CT4#121, including C4-240405, C4-240406, C4-240407, C4-240409, C4-240410, C4-240470, C4-240704, C4-240705, C4-240708, C4-240716 | 1.1.0 |
| 2024-03 | CT#103 | CP-240025 |  |  |  | TS presented for approval | 2.0.0 |
| 2024-03 | CT#103 |  |  |  |  | TS approved in CT#103 | 18.0.0 |
| 2024-06 | CT#104 | CP-241044 | 0001 |  | F | Support of SDP attributes a=3gpp-bdc-used-by and a=3gpp-req-app | 18.1.0 |
| 2024-06 | CT#104 | CP-241044 | 0002 | 1 | F | Correction on the Nimsas_MediaControl service | 18.1.0 |
| 2024-06 | CT#104 | CP-241044 | 0003 | 1 | F | Correction on the Nimsas_SessionEventControl service | 18.1.0 |
| 2024-06 | CT#104 | CP-241044 | 0004 |  | F | Correction on the Nimsas_MediaControl OpenAPI | 18.1.0 |
| 2024-06 | CT#104 | CP-241044 | 0006 | 1 | F | Update the presence condition and cardinality for mediaProxyConfig | 18.1.0 |
| 2024-06 | CT#104 | CP-241044 | 0007 | 1 | F | Update the DcMediaSpecification for Nimsas_SessionEventControl Service API | 18.1.0 |
| 2024-06 | CT#104 | CP-241044 | 0008 | 1 | F | Update the DcMediaSpecification Datatype for MdcEndpoint | 18.1.0 |
| 2024-06 | CT#104 | CP-241044 | 0009 | 1 | F | Update the SessionEventNotificationUri for session event notification | 18.1.0 |
| 2024-06 | CT#104 | CP-241044 | 0010 | 1 | F | Report the media HOLD to DCSF | 18.1.0 |
| 2024-06 | CT#104 | CP-241044 | 0011 | 2 | F | Add the Media re-negotiation indication to Nimsas_MediaControl API to support AR | 18.1.0 |
| 2024-06 | CT#104 |  | 0012 |  | F | 29.175 Rel-18 API version and External doc update | 18.1.0 |
| 2024-09 | CT#105 | CP-242048 | 0013 |  | F | Corrections and Updates to Notify Service Operation | 18.2.0 |
| 2024-09 | CT#105 | CP-242048 | 0014 |  | F | Remove the SecuritySetup in clause 6.1.6.1 | 18.2.0 |
| 2024-09 | CT#105 | CP-242048 | 0015 | 1 | F | Update the sessionInfo description for Nimsas_SessionEventControl API | 18.2.0 |
| 2024-12 | CT#106 | CP-243027 | 0017 |  | F | DC resource release due to a CANCEL request | 19.0.0 |
| 2024-12 | CT#106 | CP-243027 | 0023 | 1 | F | Remove the MRF | 19.0.0 |
| 2024-12 | CT#106 | CP-243027 | 0020 | 1 | F | Report the QoS info to DCSF | 19.0.0 |
| 2024-12 | CT#106 | CP-243068 | 0032 |  | F | API version and External doc update | 19.0.0 |
| 2024-12 | CT#106 | CP-243045 | 0019 | 1 | B | Supporting of network initiated IMS Data Channel | 19.0.0 |
| 2024-12 | CT#106 | CP-243045 | 0022 | 1 | B | Support of interworking with MTSI client | 19.0.0 |
| 2024-12 | CT#106 | CP-243045 | 0025 |  | D | Update the description of EventInitiator data type | 19.0.0 |
| 2024-12 | CT#106 | CP-243045 | 0027 |  | B | Support of the specific PSI within Called ID for standalone DC | 19.0.0 |
| 2024-12 | CT#106 | CP-243045 | 0021 | 2 | B | Definition of Nimsas_ImsSessionManagement Service | 19.0.0 |
| 2024-12 | CT#106 | CP-243045 | 0030 | 1 | B | Definition of Nimsas_ImsEE Service | 19.0.0 |
| 2024-12 | CT#106 | CP-243045 | 0024 | 2 | F | Event Notification for PS Data Off Status | 19.0.0 |
| 2024-12 | CT#106 | CP-243045 | 0028 | 2 | B | Support of IMS AS Event Exposure Service | 19.0.0 |
| 2024-12 | CT#106 | CP-243069 | 0033 |  | F | API version and External doc update | 19.0.0 |
| 2025-03 | CT#107 | CP-250047 | 0043 | 1 | B | Clarification on the MediaInfo of the IMS session management service | 19.1.0 |
| 2025-03 | CT#107 | CP-250047 | 0040 | 1 | B | Support of avatar communication | 19.1.0 |
| 2025-03 | CT#107 | CP-250047 | 0044 | 1 | B | Add Nimsas_ImsPP service to support 3rd party identity information | 19.1.0 |
| 2025-03 | CT#107 | CP-250047 | 0034 | 2 | B | Notify Service Operation of Nimsas_SessionManagement API | 19.1.0 |
| 2025-03 | CT#107 | CP-250047 | 0039 | 2 | B | Update Nimsas_ImsSessionManagement service | 19.1.0 |
| 2025-03 | CT#107 | CP-250047 | 0035 | 2 | B | Update the service description for terminating BDC establishment without DC SDP | 19.1.0 |
| 2025-03 | CT#107 | CP-250047 | 0036 | 2 | B | Update the Nimsas_SessionEventControl service to support Network Capability exposure | 19.1.0 |
| 2025-03 | CT#107 | CP-250047 | 0037 | 2 | B | Update the description of callingIdentity for Nimsas_SessionEventControl service | 19.1.0 |
| 2025-03 | CT#107 | CP-250047 | 0038 | 2 | B | Update the Nimsas_SessionEventControl service to support PS Data Off status report | 19.1.0 |
| 2025-03 | CT#107 | CP-250047 | 0045 | 2 | B | Corrections on service name and HTTP method | 19.1.0 |
| 2025-03 | CT#107 | CP-250047 | 0046 | 5 | B | Nimsas_EE subscribe operation | 19.1.0 |
| 2025-03 | CT#107 | CP-250137 | 0047 |  | F | API version and External doc update | 19.1.0 |
| 2025-06 | CT#108 | CP-251049 | 0052 | 1 | A | Correction on the MediaInstructions data type for AR media | 19.2.0 |
| 2025-06 | CT#108 | CP-251049 | 0061 | 2 | A | Clarify the calling and called party identity | 19.2.0 |
| 2025-06 | CT#108 | CP-251049 | 0065 | 3 | A | Removal of Mediain Nimsas_SessionEventControl | 19.2.0 |
| 2025-06 | CT#108 | CP-251068 | 0050 |  | D | Editorial modification on the description of AdcType | 19.2.0 |
| 2025-06 | CT#108 | CP-251068 | 0058 |  | F | Definition of media correlation ID | 19.2.0 |
| 2025-06 | CT#108 | CP-251068 | 0059 |  | B | Usage of common data types in IMS event exposure | 19.2.0 |
| 2025-06 | CT#108 | CP-251068 | 0048 | 1 | B | Update SessionEventControl Service to Notify Operation from External Party | 19.2.0 |
| 2025-06 | CT#108 | CP-251068 | 0053 | 1 | F | Update Nimsas_ImsSessionManagement service | 19.2.0 |
| 2025-06 | CT#108 | CP-251068 | 0054 | 1 | B | Updates the Nimsas_MediaControl service to support DC multiplexing | 19.2.0 |
| 2025-06 | CT#108 | CP-251068 | 0055 | 1 | B | Update the MediaProxy value | 19.2.0 |
| 2025-06 | CT#108 | CP-251068 | 0057 | 1 | F | Update the application errors for Nimsas_ImsEE service | 19.2.0 |
| 2025-06 | CT#108 | CP-251068 | 0063 | 1 | B | Remove the temporarily defined mediaInfoExternal | 19.2.0 |
| 2025-06 | CT#108 | CP-251068 | 0069 |  | F | Correction on Nimsas_ImsEE service | 19.2.0 |
| 2025-06 | CT#108 | CP-251068 | 0066 | 1 | F | Corrections on PS Data Off | 19.2.0 |
| 2025-06 | CT#108 | CP-251068 | 0067 | 1 | B | Add Nimsas_ImsEE OpenAPI | 19.2.0 |
| 2025-06 | CT#108 | CP-251068 | 0068 | 1 | B | Add Nimsas_ImsSessionManagement OpenAPI | 19.2.0 |
| 2025-06 | CT#108 | CP-251068 | 0070 | 1 | F | Correction on Nimsas_ImsSessionManagement | 19.2.0 |
| 2025-06 | CT#108 | CP-251079 | 0072 |  | F | API version and External doc update | 19.2.0 |
| 2025-09 | CT#109 | CP-252033 | 0085 | 2 | A | Missing condition for mediaInstruction | 19.3.0 |
| 2025-09 | CT#109 | CP-252050 | 0079 |  | B | Nimsas_ImsPP_Delete service operation definition | 19.3.0 |
| 2025-09 | CT#109 | CP-252050 | 0080 |  | B | Nimsas_ImsParameterProvision API: "201 Created" and "204 No Content" responses | 19.3.0 |
| 2025-09 | CT#109 | CP-252050 | 0086 |  | B | Update the service operation description | 19.3.0 |
| 2025-09 | CT#109 | CP-252050 | 0089 |  | B | Remove the editor note for Nimsas_ImsEE Service | 19.3.0 |
| 2025-09 | CT#109 | CP-252050 | 0090 |  | B | Remove the editor note in overview | 19.3.0 |
| 2025-09 | CT#109 | CP-252050 | 0073 | 1 | F | Correction on the data channel multiplexing | 19.3.0 |
| 2025-09 | CT#109 | CP-252050 | 0077 | 1 | B | Nimsas_ImsParameterProvision API: removal of EN on RCD | 19.3.0 |
| 2025-09 | CT#109 | CP-252050 | 0081 | 1 | F | Nimsas_ImsParameterProvision: correction on the Public User Identity | 19.3.0 |
| 2025-09 | CT#109 | CP-252050 | 0078 | 1 | D | Nimsas_ImsParameterProvision API: editorial corrections | 19.3.0 |
| 2025-09 | CT#109 | CP-252050 | 0082 | 1 | B | Nimsas_ImsParameterProvision API: specification of the OpenAPI file | 19.3.0 |
| 2025-09 | CT#109 | CP-252050 | 0083 | 2 | B | Instruction on transcoding for interworking | 19.3.0 |
| 2025-09 | CT#109 | CP-252050 | 0076 | 2 | F | Correction on the SessionId attribute description | 19.3.0 |
| 2025-09 | CT#109 | CP-252050 | 0092 | 2 | F | Add PUT operation in Nimsas_ImsSessionManagement_Update | 19.3.0 |
| 2025-09 | CT#109 | CP-252050 | 0091 | 2 | B | Add DC interworking indication in DcMediaSpecification | 19.3.0 |
| 2025-09 | CT#109 | CP-252176 | 0093 |  | F | API version and External doc update | 19.3.0 |