| 3GPP TS 29.176 V0.3.0 (2023-08) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; IP Multimedia Subsystems (IMS); Media Function (MF) Services; Stage 3 (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions, symbols and abbreviations	8

3.1	Definitions	8

3.2	Symbols	8

3.3	Abbreviations	8

4	Overview	8

5	Services offered by the MF	9

5.1	Introduction	9

5.2	Nmf_MediaResourceManagement (MRM) Service	9

5.2.1	Service Description	9

5.2.2	Service Operations	9

5.2.2.1	Introduction	9

5.2.2.2	Nmf_MRM_Create Service Operation	9

5.2.2.2.1	General	9

5.2.2.2.2	Creation of a new media context	10

5.2.2.3	Nmf_MRM_Update Service Operation	11

5.2.2.3.1	General	11

5.2.2.3.2	Updating an existing media context	11

5.2.2.4	Nmf_MRM_Delete Service Operation	12

5.2.2.4.1	General	12

5.2.2.4.2	Deleting an existing media context	12

6	API Definitions	13

6.1	Nmf_MRM Service API	13

6.1.1	Introduction	13

6.1.2	Usage of HTTP	13

6.1.2.1	General	13

6.1.2.2	HTTP standard headers	13

6.1.2.2.1	General	13

6.1.2.2.2	Content type	13

6.1.2.3	HTTP custom headers	13

6.1.3	Resources	14

6.1.3.1	Overview	14

6.1.3.2	Resource: Contexts Collection	14

6.1.3.2.1	Description	14

6.1.3.2.2	Resource Definition	14

6.1.3.2.3	Resource Standard Methods	15

6.1.3.2.4	Resource Custom Operations	15

6.1.3.3	Resource: Individual Context	15

6.1.3.3.1	Description	15

6.1.3.3.2	Resource Definition	15

6.1.3.3.3	Resource Standard Methods	16

6.1.3.3.4	Resource Custom Operations	17

6.1.4	Custom Operations without associated resources	17

6.1.5	Notifications	17

6.1.6	Data Model	17

6.1.6.1	General	17

6.1.6.2	Structured data types	18

6.1.6.2.1	Introduction	18

6.1.6.2.2	Type: MediaContext	18

6.1.6.2.3	Type: TerminationInfo	18

6.1.6.2.4	Type: MediaInfo	19

6.1.6.2.x	Type: DcMedia	20

6.1.6.2.y	Type: ArMedia	22

6.1.6.3	Simple data types and enumerations	22

6.1.6.3.1	Introduction	22

6.1.6.3.2	Simple data types	22

6.1.7	Error Handling	22

6.1.7.1	General	22

6.1.7.2	Protocol Errors	22

6.1.7.3	Application Errors	22

6.1.8	Feature negotiation	23

6.1.9	Security	23

6.1.10	HTTP redirection	23

Annex A (normative): OpenAPI specification	24

A.1	General	24

A.2	Ndcmf_MRM API	24

A.3	<Service 2> API	27

Annex B (informative): Change history	27

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the stage 3 protocol and data model for the Ndcmf Service Based Interface. It provides stage 3 protocol definitions and message flows, and specifies the API for each service offered by the Data Channel Media Function (DCMF).

The 5G System stage 2 architecture and procedures are specified in TS 23.501 [2] and TS 23.502 [3].

The IP Multimedia Subsystem (IMS) Data Channel (DC) architecture and procedures are specified in annex AC of TS 23.228 [14].

The Technical Realization of the Service Based Architecture and the Principles and Guidelines for Services Definition are specified in TS 29.500 [4] and TS 29.501 [5].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	OpenAPI: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[7]	3GPP TR 21.900: "Technical Specification Group working methods".

[8]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[9]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[10]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[11]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[12]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[13]	IETF RFC 7807: "Problem Details for HTTP APIs".

[14]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[15]	IETF RFC 6902: " JavaScript Object Notation (JSON) Patch".

[16]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces Stage 3".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

DC2	 Reference point between MF and IMS AS

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AR	Augmented Reality

DC	Data Channel

MF	Media Function

IMS AS	IP Multimedia Subsystem Application Server

# 4	Overview

Within the IMS DC architecture, the MF offers services to the IMS AS via the Nmf service based interface (see 3GPP TS 23.228 [14]).

Figure 4.1-1 provides the reference model (in service-based interface representation and in reference point representation), with focus on the MF services.

Figure 4.1-1: Reference model – MF

DC2 is the reference point between MF and IMS AS.

# 5	Services offered by the MF

## 5.1	Introduction

The MF offers the following services via the Nmf interface:

-	Nmf_MediaResourceManagement (MRM) Service

Table 5.1-1 summarizes the corresponding APIs defined for this specification.

Table 5.1-1: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |
| Nmf_MRM | 6.1 | Nmf Media Resource Management Service | TS29176_Nmf_MRM.yaml | nmf-mrm | A.2 |

## 5.2	Nmf_MediaResourceManagement (MRM) Service

### 5.2.1	Service Description

The Nmf_MRM service as defined in 3GPP TS 23.228 [14] is provided by the Media Function (MF). This service enables the consumer to create, update and delete media resources. Data Channel (DC) and Augmented Reality (AR) are two capabilities supported by MF.

The media resource represents a media context including one or multiple media terminations. A media termination includes media resources for one or multiple medias on the Mb interface. When a media pass through the MF, there is one termination for the input stream and one termination for the output stream.

### 5.2.2	Service Operations

#### 5.2.2.1	Introduction

The Nmf_MRM service supports the following service operation.

Table 5.2.2.1-1: Service operations supported by the Nmf_MRM service

| Service Operations | Description | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Create | Create a new media context including one or multiple media terminations. | Request/Response | IMS AS |
| Update | Update the one or multiple existing media resources within a specific media context. | Request/Response | IMS AS |
| Delete | Delete a specific media context including all the existing terminations and medias. | Request/Response | IMS AS |

#### 5.2.2.2	Nmf_MRM_Create Service Operation

##### 5.2.2.2.1	General

The Nmf_MRM_Create service operation is used by an NF service consumer to create a media context including one or multiple terminations and reserve media resources for anchoring one or multiple medias of Mb interface in each termination on MF. The consumer NF may also include application function (e.g. DCSF, DC AS) specification information requested by the application layer to be applied on the media by the MF.

##### 5.2.2.2.2	Creation of a new media context

The NF service consumer shall request a new context by using HTTP method POST with "{apiRoot}/nmf/<apiVersion>/contexts" as resource URI representing the "Contexts Collection", see clause 6.1.3.2.

Figure 5.2.2.2.2-1 Creation of a media context

1.	The NF Service Consumer shall send a POST request to create an "Individual Context" resource in the MF. The payload body of the POST request shall contain a representation of the individual context resource to be created.

The NF service consumer shall include the following information in the HTTP message body:

-	List of termination descriptors;

-	List of media steam descriptors;

-	Media ID, i.e. a unique identity of the media stream within the media context instance;

-	Remote Mb specifications, i.e. the media stream IP address and port allocated at the remote endpoint, i.e. remote UE, remote network.

-	Media resource description, which includes

-	Media resource type, e.g. DC, AR.

-	If media resource type is set to "DC", the DC media specification shall be included. The DC media specification shall include:

-	Data Channel Mapping and Configuration information when originating/terminating data channel media flows on the Mb interface.

-	SCTP stream Id for the DC;

-	subprotocol, order, maxRetry, maxTime and priority may be included.

-	Maximum Message Size, which represents the maximum size to be expected.

-	Data Channel Port, which represents the port of SCTP port for the Data Channel.

-	Security Setup, which identifies the security setup of the DTLS connection.

-	Security Certificate Fingerprint, which identifies the security certificate fingerprint.

-	Security Transport Identity, which identifies transport layer identity.

For establishing bootstrap data channel or P2A/A2P application data channel, the following parameters shall be included:

-	media proxy configuration applicable to the media flow;

-	remote MDC1/MDC2 media specification information to be applied on the media by the MF;

-	Replacement HTTP URL for each streamId allocated by the application layer representing the application list (e.g. graphical user interface) offered to the IMS subscriber via the MDC1 interface.

-	If media resource type is set to "AR", the AR media resource description shall be included, which includes:

-	Media processing specification.

2a.	Upon the reception of the HTTP POST request, if the request is accepted and no error occur, the MF shall:

-	create a new media context;

-	assign a media contextId;

-	assign a terminationId for each termination descriptor; and

-	reserve media resources for each mediaId.

The MF shall include a HTTP Location header to provide the location of a newly created resource (MediaContext) together with the status code 201 indicating the requested resource is created in the response message.

2b.	On failure, one of the HTTP status code listed in Table 6.1.3.2.3.1-3 shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.3.2.3.1-3.

#### 5.2.2.3	Nmf_MRM_Update Service Operation

##### 5.2.2.3.1	General

The Nmf_MRM_Update service operation is used by an NF service consumer to update one or multiple existing media resources within a specific media context on MF. Terminations and/or Medias can be added, modified, or deleted within an Update request.

##### 5.2.2.3.2	Updating an existing media context

The NF Service Consumer shall modify the context by using HTTP method PATCH with the URI "{apiRoot}/nmf/<apiVersion>/contexts/{contextId}" as resource URI representing the "Individual Context ", see clause 6.1.3.3.

Figure 5.2.2.3.2-1 Update a media context

1.	The NF Service Consumer shall send a PATCH request to modify a context resource in the MF. The modification may be for adding, updating or deleting terminations of the existing media context.

For adding, removing or updating a termination in an existing MediaContext, the payload body of the PATCH request shall contain an "add", "remove" or "replace" PATCH operation respectively, with one item of the attribute "terminations" of the MediaContext.

2a.	On success, if the change is to delete the existing termination and MF accepts the request change, the MF shall return the status code 204 No Content.

2b. On success, if the change is to add a new termination or to update the existing termination and DCMF accepts the request change, the MF shall return the status code 200 OK. The response shall contain the new resource representation of the resource Individual Context, which includes the added and modified resource or its sub-resource.

2c	On failure, one of the HTTP status code listed in Table 6.1.3.3.3.1-3 shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in Table 6.1.3.3.3.1-3.

#### 5.2.2.4	Nmf_MRM_Delete Service Operation

##### 5.2.2.4.1	General

The Nmf_MRM_Delete service operation is used by an NF service consumer to delete an existing media context including all existing terminations and medias on MF previously created by itself.

##### 5.2.2.4.2	Deleting an existing media context

The NF Service Consumer shall delete an existing context by using HTTP method DELETE with the URI "{apiRoot}/nmf/<apiVersion>/contexts/{contextId}" as resource URI representing the "Individual Context ", see clause 6.1.3.3.

Figure 5.2.2.4.2-1 Delete a media context

1.	The NF Service Consumer shall send a DELETE request to delete an existing context resource in the MF.

2a.	On success, the request is accepted, the MF shall reply with the status code 204 No Content indicating the resource identified by context ID is successfully deleted in the response message.

2b.	On failure, one of the HTTP status code listed in Table 6.1.3.3.3.2-3 shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in Table 6.1.3.3.3.2-3.

# 6	API Definitions

## 6.1	Nmf_MRM Service API

### 6.1.1	Introduction

The Nmf_MRM service shall use the Ndcmf_MRM API.

The API URI of the Nmf_MRM API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests from the NF service consumer towards the NF service producer shall have the Resource URI structure defined in clause 4.4.1 of 3GPP TS 29.501 [5], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [5].

-	The <apiName> shall be " nmf_mrm".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 6.1.3.

### 6.1.2	Usage of HTTP

#### 6.1.2.1	General

HTTP/2, IETF RFC 7540 [11], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

The OpenAPI [6] specification of HTTP messages and content bodies for the Nmf_MRM API is contained in Annex A.

#### 6.1.2.2	HTTP standard headers

##### 6.1.2.2.1	General

See clause 5.2.2 of 3GPP TS 29.500 [4] for the usage of HTTP standard headers.

##### 6.1.2.2.2	Content type

JSON, IETF RFC 8259 [12], shall be used as content type of the HTTP bodies specified in the present specification as specified in clause 5.4 of 3GPP TS 29.500 [4]. The use of the JSON format shall be signalled by the content type "application/json".

"Problem Details" JSON object shall be used to indicate additional details of the error in a HTTP response body and shall be signalled by the content type "application/problem+json", as defined in IETF RFC 7807 [13].

The use of JSON Patch (IETF RFC 6902 [15]) format in a HTTP request body shall be signaled by the content type "application/json-patch+json".

#### 6.1.2.3	HTTP custom headers

The mandatory HTTP custom header fields specified in clause 5.2.3.2 of 3GPP TS 29.500 [4] shall be supported, and the optional HTTP custom header fields specified in clause 5.2.3.3 of 3GPP TS 29.500 [4] may be supported.

### 6.1.3	Resources

#### 6.1.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.1.3.1-1 depicts the resource URIs structure for the Nmf_MRM API.

Figure 6.1.3.1-1: Resource URI structure of the Nmf_MRM API

Table 6.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.1.3.1-1: Resources and methods overview

| Resource purpose/name | Resource URI (relative path after API URI) | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Contexts Collection | /contexts | POST | Creates a new Individual Context resource. |
| Individual Context | /contexts/{contextId} | PATCH | Updates an existing Individual Context resource identified by {contextId} |
|  |  | DELETE | Deletes an existing Individual Context resource identified by {contextId}. |

#### 6.1.3.2	Resource: Contexts Collection

##### 6.1.3.2.1	Description

The Contexts Collection resource represents a collection of contexts created by NF service consumers of Nmf_MRM service at a given MF. The resource is modelled as Collection resource archetype (see clause C.2 of 3GPP TS 29.501 [2]).

##### 6.1.3.2.2	Resource Definition

Resource URI: {apiRoot}/ nmf_mrm/<apiVersion>/contexts

This resource shall support the resource URI variables defined in table 6.1.3.2.2-1.

Table 6.1.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1 |

##### 6.1.3.2.3	Resource Standard Methods

6.1.3.2.3.1	POST

This method shall support the URI query parameters specified in table 6.1.3.2.3.1-1.

Table 6.1.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.3.2.3.1-3.

Table 6.1.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MediaContext | M | 1 | Creates a new Individual Context resource |

Table 6.1.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MediaContext | M | 1 | 201 Created | The creation of an Individual Media Context resource is confirmed and a representation of that resource is returned. |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 6.1.3.2.3.1-4: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/nmf-mrm/<apiVersion>/contexts/{contextId}. |

##### 6.1.3.2.4	Resource Custom Operations

None in the release of this specification.

#### 6.1.3.3	Resource: Individual Context

##### 6.1.3.3.1	Description

The Individual Context resource represents an individual context created by the NF service consumers of Nmf_MRM service at a given MF. This resource is modelled as the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [2]).

##### 6.1.3.3.2	Resource Definition

Resource URI: {apiRoot}/nmf_mrm/<apiVersion>/contexts/{contextId}

The <apiVersion> shall be set as described in clause 6.3.1.

This resource shall support the resource URI variables defined in table 6.1.3.3.2-1.

Table 6.1.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1 |
| contextId | string | Identifies an individual context to the Nmf_MRM service. |

##### 6.1.3.3.3	Resource Standard Methods

6.1.3.3.3.1	PATCH

This method shall support the URI query parameters specified in table 6.1.3.3.3.1-1.

Table 6.1.3.3.3.1-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.1-2 and the response data structures and response codes specified in table 6.1.3.3.3.1-3.

Table 6.1.3.3.3.1-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(PatchItem) | M | 1..N | Document describes the modification(s) to an Individual Context resource. |

Table 6.1.3.3.3.1-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MediaContext | M | 1..N | 200 OK | Represents a successful update on the media context. |
| n/a |  |  | 204 No Content |  |
| ProblemDetails | O | 0..1 | 403 Forbidden | Indicates the modification of media context has failed due to application error. The "cause" attribute may be used to indicate one of the following application errors: - MEDIA_ID_CONFLICT - MEDIA_CONNECTION_CHANGED |
| ProblemDetails | O | 0..1 | 404 Not Found | Indicates the modification of media context has failed due to application error. The "cause" attribute may be used to indicate one of the following application errors: - CONTEXT_NOT_FOUND |
| NOTE 1: The manadatory HTTP error status code for the PATCH method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

6.1.3.3.3.2	DELETE

This method shall support the URI query parameters specified in table 6.1.3.3.3.2-1.

Table 6.1.3.3.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.2-2 and the response data structures and response codes specified in table 6.1.3.3.3.2-3.

Table 6.1.3.3.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.3.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No content |  |
| Problem Details | O | 0..1 | 404 Not Found | Indicates the deletion of the context has failed due to application error. The "cause" attribute may be used to indicate one of the following application errors: - CONTEXT_NOT_FOUND |
| NOTE 1: The manadatory HTTP error status code for the DELETE method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

##### 6.1.3.3.4	Resource Custom Operations

None in the release of specification.

### 6.1.4	Custom Operations without associated resources

None in this release of specification.

### 6.1.5	Notifications

None in this release of specification.

### 6.1.6	Data Model

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nmf_MRM  service based interface protocol.

Table 6.1.6.1-1: Nmf_MRM specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| MediaContext | 6.1.6.2.2 | Represents an individual context resource on MF. |  |
| TerminationInfo | 6.1.6.2.3 | Represents the termination information. |  |
| MediaInfo | 6.1.6.2.4 | Represents the media Information. |  |
| DcMedia | 6.1.6.2.5 | Represents the DC media descriptor. |  |
| ArMedia | 6.1.6.2.6 | Represents the AR media descriptor. |  |

Table 6.1.6.1-2 specifies data types re-used by the Nmf_MRM service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nmf_MRM service based interface.

Table 6.1.6.1-2: Nmf_MRM re-used Data Types

| Data type | Reference |  |  | Comments |  |  | Applicability |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ProblemDetails |  | 3GPP TS 29.571 [16] |  |  | Problem Details |  |  |  |  |
| IpEndpoint |  |  | 3GPP TS 29.510 [10] |  |  | IP endpoint |  |  |  |
| MediaId |  |  | 3GPP TS 29.571 [16] |  |  | Uniquely identifies one media flow within an IMS session. |  |  |  |
| MaxMessageSize |  |  | 3GPP TS 29.571 [16] |  |  | Max SCTP user message size. |  |  |  |
| MediaResourceType |  |  | 3GPP TS 29.571 [16] |  |  | Media resource types. |  |  |  |
| MediaProxy |  |  | 3GPP TS 29.571 [16] |  |  | Media proxy configuration applicable to the media flow |  |  |  |
| SecuritySetup |  |  | 3GPP TS 29.571 [16] |  |  | Security setup of the DTLS association. |  |  |  |
| DcEndpoint |  |  | 3GPP TS 29.571 [16] |  |  | DC endpoint, which includes |  |  |  |
| DcStream |  |  | 3GPP TS 29.571 [16] |  |  | Data channel stream information. |  |  |  |
| ReplaceHttpUrl |  |  | 3GPP TS 29.571 [16] |  |  | Replacement HTTP URL allocated by the application layer. |  |  |  |

#### 6.1.6.2	Structured data types

##### 6.1.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 6.1.6.2.2	Type: MediaContext

Table 6.1.6.2.2-1: Definition of type MediaContext

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| contextId | string | C | 0..1 | Identifies a specific context. It is assigned by the MF when creating a new context, i.e. Nmf_MRM_Create operation. |  |
| terminations | array(TerminationInfo) | M | 1..N | Represents the media terminations |  |

##### 6.1.6.2.3	Type: TerminationInfo

Table 6.1.6.2.3-1: Definition of type TerminationInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| terminationId | string | C | 0..1 | Identifies a specific media termination. It is assigned by the MF when creating a new termination. It shall be included in the response of adding a new media termination, i.e., the Nmf_MRM_Create operation and in the Nmf_MRM_Update operation with adding a new termination. |  |
| medias | array(MediaInfo) | M | 1..N | Represent the list of media resources for one or multiple medias on the Mb interface. |  |

##### 6.1.6.2.4	Type: MediaInfo

Table 6.1.6.2.4-1: Definition of type MediaInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mediaId | string | M | 1 | Identifies a media stream, e.g. data channel. |  |
| mediaResourceType | MediaResourceType | M | 1 | Represents the media resource type, e.g. DC, AR. |  |
| localMbEndpoint | IpEndpoint | C | 0..1 | Represents the local IP address and port number of the local endpoint of MF on the Mb interface. It will be allocated by MF. It shall be contained in the response when creating a new media stream, i.e. Nmf_MRM_Create operation or Nmf_MRM_Update operation with adding a new termination or adding a media in an existing termination. (NOTE) |  |
| remoteMbEndpoint | IpEndpoint | C | 0..1 | Represents the IP address and port number of the remote endpoint on the Mb interface, e.g. remote UE. It shall be contained in the request of creating a new media stream, i.e. Nmf_MRM_Create operation or Nmf_MRM_Update operation with adding a new termination or adding a media in an existing termination. It can be Null when originating a new media. (NOTE) |  |
| dcMedia | DcMedia | C | 0..1 | Represents the data channel media descriptors. It shall be contained if the mediaResourceType is set to "DC". |  |
| arMedia | ArMedia | C | 0..1 | Represents the AR media descriptors. It shall be contained if the mediaResourceType is set to "AR". |  |
| NOTE: The IE cannot be changed once the media has been established. |  |  |  |  |  |

##### 6.1.6.2.x	Type: DcMedia

Table 6.1.6.2.x-1: Definition of type DcMedia

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mediaProxyConfig | MediaProxy | C | 0..1 | Represents the media proxy configuration. It shall be included if the mediaId reprensents bootstrap or P2A/A2P application data channels. |  |
| replaceHttpUrl | ReplacementHttpUrl | C | 0..2 | Represents URL for the specific IMS subscriber when requesting the application list (e.g. graphical user interface) via the MDC1 interface. It shall be included if the mediaId represents bootstrap data channel and streamId is 0 and 100. |  |
| remoteMdc1Endpoint | IpEndpoint | C | 0..1 | Represents the remote MDC1 media endpoint information, i.e. the IP address and port number of DCSF. It shall be included if the mediaId represents a bootstrap data channel. |  |
| remoteMdc2Endpoint | IpEndpoint | C | 0..1 | Represents the remote MDC2 media endpoint address, i.e. the IP address and port number of DC AS. It shall be included if the mediaId represents a P2A/A2P application data channel and the mediaProxyConfig is set to "HTTP". |  |
| localMdc1Mdc2Endpoint | IpEndpoint | C | 0..1 | Represents the local MF MDC1/MDC2 media endpoint information. It will be allocated by MF and contained in the response when the mediaId represents a P2A/A2P application data channel and bootstrap data channel. |  |
| mdc2Protocol | string | O | 0..1 | Represents the transport layer protocols for MDC2 interface. It may be included when mediaProxyConfig is set "HTTP" and remoteMdc2Endpoint is present. This IE is formatted as the following pattern, for example: 'UDP/DTLS/SCTP' |  |
| streams | array(DcStream) | M | 1..N | Represents the data channel mapping and configuration information when originating/terminating data channel media flows on the Mb interface. |  |
| maxMessageSize | MaxMessageSize | O | 0..1 | Represents the maximum size of to be expected. |  |
| localDcEndpoint | DcEndpoint | C | 0..1 | Represents the DC endpoint for the Data Channel. It will be allocated by MF. It shall be contained in the response when creating a new media stream, i.e. Nmf_MRM_Create operation or Nmf_MRM_Update operation with adding a new termination or adding a media in an existing termination. (NOTE) |  |
| remoteDcEndpoint | DcEndpoint | C | 0..1 | Represents the DC Endpoint for the Data Channel on the Mb interface. It shall be contained in the request when creating a new media stream , i.e. Nmf_MRM_Create operation or Nmf_MRM_Update operation with adding a new termination or adding a media in an existing termination. (NOTE) |  |
| securitySetup | SecuritySetup | O | 0..1 | Represents the security set up of the DTLS association. |  |
| NOTE: The IE cannot be changed once the media has been established. |  |  |  |  |  |

##### 6.1.6.2.y	Type: ArMedia

Table 6.1.6.2.y-1: Definition of type ArMedia

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |

Editor’s note: The definition of ArMedia is FFS.

#### 6.1.6.3	Simple data types and enumerations

##### 6.1.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 6.1.6.3.2	Simple data types

The simple data types defined in table 6.1.6.3.2-1 shall be supported.

Table 6.1.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

### 6.1.7	Error Handling

#### 6.1.7.1	General

For the Nmf_MRM API, HTTP error responses shall be supported as specified in clause 4.8 of 3GPP TS 29.501 [5]. Protocol errors and application errors specified in table 5.2.7.2-1 of 3GPP TS 29.500 [4] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.2.7.1-1 of 3GPP TS 29.500 [4].

In addition, the requirements in the following clauses are applicable for the Nmf_MRM API.

#### 6.1.7.2	Protocol Errors

No specific procedures for the Nmf_MRM service are specified.

#### 6.1.7.3	Application Errors

The application errors defined for the Nmf_MRM service are listed in Table 6.1.7.3-1.

Table 6.1.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| CONTEXT_NOT_FOUND | 404 Not Found | Indicates that the requested context is not found in the MF. |
| MEDIA_ID_CONFLICT | 403 Forbidden | Indicates that mediaId which identifies the newly added media has existed within the specific termination. |
| MEDIA_CONNECTION_CHANGED | 403 Forbidden | Indicates that the connection information which has been marked that cannot be changed once the media has established is changed in the request. including the remoteMbEndpoint, localMbEndpoint etc. |

### 6.1.8	Feature negotiation

The optional features in table 6.1.8-1 are defined for the Nmf_MRM API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 6.1.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 6.1.9	Security

As indicated in 3GPP TS 33.501 [8] and 3GPP TS 29.500 [4], the access to the Nmf_MRM API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [9]), based on local configuration, using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [10]) plays the role of the authorization server.

If OAuth2 is used, an NF Service Consumer, prior to consuming services offered by the Nmf_MRM API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [10], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Nmf_MRM service.

The Nmf_MRM API defines a single scope " nmf-mrm" for the entire service, and it does not define any additional scopes at resource or operation level.

### 6.1.10	HTTP redirection

An HTTP request may be redirected to a different <NF> service instance when using direct or indirect communications (see 3GPP TS 29.500 [4]).

An SCP that reselects a different <NF> producer instance will return the NF Instance ID of the new <NF> producer instance in the 3gpp-Sbi-Producer-Id header, as specified in clause 6.10.3.4 of 3GPP TS 29.500 [4].

If an <NF> redirects a service request to a different <NF> using an 307 Temporary Redirect or 308 Permanent Redirect status code, the identity of the new <NF> towards which the service request is redirected shall be indicated in the 3gpp-Sbi-Target-Nf-Id header of the 307 Temporary Redirect or 308 Permanent Redirect response as specified in clause 6.10.9.1 of 3GPP TS 29.500 [4].

Annex A (normative):
OpenAPI specification

# A.1	General

This Annex specifies the formal definition of the API(s) defined in the present specification. It consists of OpenAPI specifications in YAML format.

This Annex takes precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API(s).

NOTE 1:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification files contained in this 3GPP Technical Specification are available on a Git-based repository that uses the GitLab software version control system (see clause 5.3.1 of 3GPP TS 29.501 [5] and clause 5B of 3GPP TR 21.900 [7]).

# A.2	Ndcmf_MRM API

One clause is introduced per Service, with the corresponding OpenAPI 3.0.0 Document.

Rapporteur of a new TS, which is based on this template shall inset a correct year below (see © <yyyy>). The rapporteur shall also update the year, if changes on the OpenAPI are agreed in a different year.

openapi: 3.0.0

info:

title: <API Name>

version: 1.0.0-alpha.1

description: |

<API Name> Service.

© <yyyy>, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.<xxx> V<x.y.z>; <TS Name>.

url: http://www.3gpp.org/ftp/Specs/archive/29_series/29.xxx/

servers:

- url: '{apiRoot}/<API name in lower letters. Composed names are separated with a hyphen>/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- <API name in lower letters with undesrscores>

paths:

# API specific definitions , below is an example

/subscriptions:

post:

summary: subscribe to notifications

operationId: CreateIndividualSubcription

tags:

- Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NsmfEventExposure'

responses:

'201':

description: Success

content:

application/json:

schema:

$ref: '#/components/schemas/<xxx>'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/<API name in lower letters. Composed names are separated with a hyphen>/<version>/subscriptions/{subId}'

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

myNotification:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/<yyy>'

responses:

'204':

description: No Content, Notification was succesfull

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscriptions/{subId}:

get:

summary: retrieve subscription

operationId: GetIndividualSubcription

tags:

- IndividualSubscription (Document)

parameters:

- name: subId

in: path

description: Event Subscription ID

required: true

schema:

type: string

responses:

'200':

description: OK. Resource representation is returned

content:

application/json:

schema:

$ref: '#/components/schemas/<xxx>'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

put:

summary: update subscription

operationId: ReplaceIndividualSubcription

tags:

- IndividualSubscription (Document)

tags:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/<xxx>'

parameters:

- name: subId

in: path

description: Event Subscription ID

required: true

schema:

type: string

responses:

'200':

description: OK. Resource was succesfully modified and representation is returned

content:

application/json:

schema:

$ref: '#/components/schemas/<xxx>'

'204':

description: No Content. Resource was succesfully modified

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: unsubscribe from notifications

operationId: DeleteIndividualSubcription

tags:

- IndividualSubscription (Document)

parameters:

- name: subId

in: path

description: Event Subscription ID

required: true

schema:

type: string

responses:

'204':

description: No Content. Resource was succesfully deleted

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

<API name in lower letters . Composed names are separated with a hyphen>: Access to the <API Name> API

# API specific definitions

# A.3	<Service 2> API

And so on if there are more than two services supported by the NF.

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2023-04 | CT4#115e | C4-231552 |  |  |  | Version 0.1.0 | 0.1.0 |
| 2023-05 | CT4#116 | C4-232178 |  |  |  | Clause 4 Overview | 0.2.0 |
| 2023-05 | CT4#116 | C4-232224 |  |  |  | Clause 5.1 Introduction of DCMF services | 0.2.0 |
| 2023-05 | CT4#116 | C4-232506 |  |  |  | Service description and service operation introduction of MRM service | 0.2.0 |
| 2023-05 | CT4#116 | C4-232607 |  |  |  | MRM API definition | 0.2.0 |
| 2023-05 | CT4#116 | C4-232637 |  |  |  | MRM service operations | 0.2.0 |
| 2023-08 | CT4#117 | C4-233707 |  |  |  | Change DCMF to MF | 0.3.0 |
| 2023-08 | CT4#117 | C4-233307 |  |  |  | Update the data channel related data types | 0.3.0 |
| 2023-08 | Ct4#117 | C4-233306 |  |  |  | Update MF service to support AR | 0.3.0 |