| 3GPP TS 29.505 V17.2.0 (2021-03) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; 5G System; Usage of the Unified Data Repository services for Subscription Data; Stage 3 (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

# Contents

Foreword	10

1	Scope	11

2	References	11

3	Definitions and abbreviations	12

3.1	Definitions	12

3.2	Abbreviations	12

4	Overview	12

5	Usage of Nudr_DataRepository Service	12

5.1	Introduction	12

5.2	Resources	13

5.2.1	Overview	13

5.2.2	Resource: AuthenticationSubscription	21

5.2.2.1	Description	21

5.2.2.2	Resource Definition	21

5.2.2.3	Resource Standard Methods	22

5.2.2.3.1	GET	22

5.2.2.3.2	PATCH	22

5.2.3	Resource: AccessAndMobilitySubscriptionData	23

5.2.3.1	Description	23

5.2.3.2	Resource Definition	23

5.2.3.3	Resource Standard Methods	23

5.2.3.3.1	GET	23

5.2.4	Resource: SmfSelectionSubscriptionData	24

5.2.4.1	Description	24

5.2.4.2	Resource Definition	24

5.2.4.3	Resource Standard Methods	24

5.2.4.3.1	GET	24

5.2.5	Resource: SessionManagementSubscriptionData	25

5.2.5.1	Description	25

5.2.5.2	Resource Definition	25

5.2.5.3	Resource Standard Methods	25

5.2.5.3.1	GET	25

5.2.6	Resource: Amf3GppAccessRegistration	26

5.2.6.1	Description	26

5.2.6.2	Resource Definition	26

5.2.6.3	Resource Standard Methods	27

5.2.6.3.1	PUT	27

5.2.6.3.2	PATCH	27

5.2.6.3.3	GET	28

5.2.7	Resource: AmfNon3GppAccessRegistration	28

5.2.7.1	Description	28

5.2.7.2	Resource Definition	29

5.2.7.3	Resource Standard Methods	29

5.2.7.3.1	PUT	29

5.2.7.3.2	PATCH	29

5.2.7.3.3	GET	30

5.2.8	Resource: SmfRegistrations	30

5.2.8.1	Description	30

5.2.8.2	Resource Definition	31

5.2.8.3	Resource Standard Methods	31

5.2.8.3.1	GET	31

5.2.9	Resource: IndividualSmfRegistration	31

5.2.9.1	Description	31

5.2.9.2	Resource Definition	31

5.2.9.3	Resource Standard Methods	32

5.2.9.3.1	PUT	32

5.2.9.3.2	DELETE	32

5.2.9.3.3	GET	33

5.2.10	Resource: OperatorSpecificData	33

5.2.10.1	Description	33

5.2.10.2	Resource Definition	33

5.2.10.3	Resource Standard Methods	34

5.2.10.3.1	GET	34

5.2.10.3.2	PATCH	34

5.2.11	Resource: Smsf3GppAccessRegistration	35

5.2.11.1	Description	35

5.2.11.2	Resource Definition	35

5.2.11.3	Resource Standard Methods	35

5.2.11.3.1	PUT	35

5.2.11.3.2	DELETE	36

5.2.11.3.4	GET	36

5.2.12	Resource: SmsfNon3GppAccessRegistration	37

5.2.12.1	Description	37

5.2.12.2	Resource Definition	37

5.2.12.3	Resource Standard Methods	37

5.2.12.3.1	PUT	37

5.2.12.3.2	DELETE	38

5.2.12.3.4	GET	38

5.2.12A	Resource: IpSmGwRegistration	39

5.2.12A.1	Description	39

5.2.12A.2	Resource Definition	39

5.2.12A.3	Resource Standard Methods	39

5.2.12A.3.1	PUT	39

5.2.12A.3.2	DELETE	40

5.2.12A.3.3	PATCH	40

5.2.12A.3.4	GET	41

5.2.12B	Resource: MessageWaitingData	41

5.2.12B.1	Description	41

5.2.12B.2	Resource Definition	42

5.2.12B.3	Resource Standard Methods	42

5.2.12B.3.1	PUT	42

5.2.12B.3.2	DELETE	42

5.2.12B.3.3	PATCH	43

5.2.12B.3.4	GET	43

5.2.13	Resource: SMSManagementSubscriptionData	44

5.2.13.1	Description	44

5.2.13.2	Resource Definition	44

5.2.13.3	Resource Standard Methods	44

5.2.13.3.1	GET	44

5.2.14	Resource: ProvisionedParamenterData	45

5.2.14.1	Description	45

5.2.14.2	Resource Definition	45

5.2.14.3	Resource Standard Methods	45

5.2.14.3.1	GET	45

5.2.14.3.2	PATCH	46

5.2.15	Resource: SMSSubscriptionData	46

5.2.15.1	Description	46

5.2.15.2	Resource Definition	46

5.2.15.3	Resource Standard Methods	47

5.2.15.3.1	GET	47

5.2.16	Resource: SdmSubscriptions	47

5.2.16.1	Description	47

5.2.16.2	Resource Definition	47

5.2.16.3	Resource Standard Methods	48

5.2.16.3.1	GET	48

5.2.16.3.2	POST	48

5.2.17	Resource: IndividualSdmSubscription	49

5.2.17.1	Description	49

5.2.17.2	Resource Definition	49

5.2.17.3	Resource Standard Methods	49

5.2.17.3.1	PUT	49

5.2.17.3.2	DELETE	49

5.2.17.3.3	PATCH	50

5.2.17.3.4	GET	51

5.2.18	Resource: EeSubscriptions	51

5.2.18.1	Description	51

5.2.18.2	Resource Definition	52

5.2.18.3	Resource Standard Methods	52

5.2.18.3.1	GET	52

5.2.18.3.2	POST	52

5.2.19	Resource: IndividualEeSubscription	53

5.2.19.1	Description	53

5.2.19.2	Resource Definition	53

5.2.19.3	Resource Standard Methods	53

5.2.19.3.1	PUT	53

5.2.19.3.2	DELETE	54

5.2.19.3.3	PATCH	54

5.2.19.3.4	GET	55

5.2.19A	Resource: IndividualEeGroupSubscription	55

5.2.19A.1	Description	55

5.2.19A.2	Resource Definition	55

5.2.19A.3	Resource Standard Methods	56

5.2.19A.3.1	PUT	56

5.2.19A.3.2	DELETE	56

5.2.19A.3.3	PATCH	57

5.2.19A.3.4	GET	57

5.2.20	Resource: SubscriptionDataSubscriptions	58

5.2.20.1	Description	58

5.2.20.2	Resource Definition	58

5.2.20.3	Resource Standard Methods	58

5.2.20.3.1	POST	58

5.2.20.3.2	GET	59

5.2.20.3.3	DELETE	59

5.2.21	Resource: IndividualSubscriptionDataSubscription	60

5.2.21.1	Description	60

5.2.21.2	Resource Definition	60

5.2.21.3	Resource Standard Methods	61

5.2.21.3.1	DELETE	61

5.2.21.3.2	PATCH	61

5.2.21.3.3	GET	62

5.2.22	Resource: TraceData	62

5.2.22.1	Description	62

5.2.22.2	Resource Definition	62

5.2.22.3	Resource Standard Methods	63

5.2.3.3.1	GET	63

5.2.23	Resource: IdentityData	63

5.2.23.1	Description	63

5.2.23.2	Resource Definition	63

5.2.23.3	Resource Standard Methods	64

5.2.23.3.1	GET	64

5.2.24	Resource: AuthenticationStatus	64

5.2.24.1	Description	64

5.2.24.2	Resource Definition	64

5.2.24.3	Resource Standard Methods	64

5.2.24.3.1	PUT	64

5.2.24.3.2	GET	65

5.2.24.3.3	DELETE	65

5.2.24A	Resource: IndividualAuthenticationStatus	66

5.2.24A.1	Description	66

5.2.24A.2	Resource Definition	66

5.2.24A.3	Resource Standard Methods	66

5.2.24A.3.1	PUT	66

5.2.24A.3.2	GET	67

5.2.24A.3.3	DELETE	67

5.2.25	Resource: AuthenticationSoR	68

5.2.25.1	Description	68

5.2.25.2	Resource Definition	68

5.2.25.3	Resource Standard Methods	68

5.2.25.3.1	PUT	68

5.2.25.3.2	GET	68

5.2.25A	Resource: AuthenticationUPU	69

5.2.25A.1	Description	69

5.2.25A.2	Resource Definition	69

5.2.25A.3	Resource Standard Methods	69

5.2.25A.3.1	PUT	69

5.2.25A.3.2	GET	70

5.2.25B	Resource: SubscribedSNSSAIs	70

5.2.25B.1	Description	70

5.2.25B.2	Resource Definition	70

5.2.25B.3	Resource Standard Methods	71

5.2.25B.3.1	PUT	71

5.2.25B.3.2	GET	71

5.2.25C	Resource: SubscribedCAG	72

5.2.25C.1	Description	72

5.2.25C.2	Resource Definition	72

5.2.25C.3	Resource Standard Methods	72

5.2.25C.3.1	PUT	72

5.2.25C.3.2	GET	73

5.2.26	Resource: ProvisionedData	73

5.2.26.1	Description	73

5.2.26.2	Resource Definition	73

5.2.26.3	Resource Standard Methods	73

5.2.26.3.1	GET	73

5.2.27	Resource: OperatorDeterminedBarringData	74

5.2.27.1	Description	74

5.2.27.2	Resource Definition	74

5.2.27.3	Resource Standard Methods	74

5.2.27.3.1	GET	74

5.2.28	Resource: EeProfileData	75

5.2.28.1	Description	75

5.2.28.2	Resource Definition	75

5.2.28.3	Resource Standard Methods	75

5.2.28.3.1	GET	75

5.2.29	Resource: EeGroupSubscriptions	76

5.2.29.1	Description	76

5.2.29.2	Resource Definition	76

5.2.29.3	Resource Standard Methods	76

5.2.29.3.1	GET	76

5.2.29.3.2	POST	77

5.2.30	Resource: SharedData	77

5.2.30.1	Description	77

5.2.30.2	Resource Definition	77

5.2.30.3	Resource Standard Methods	77

5.2.30.3.1	GET	77

5.2.31	Resource: AmfSubscriptionInfo	78

5.2.31.1	Description	78

5.2.31.2	Resource Definition	78

5.2.31.3	Resource Standard Methods	78

5.2.31.3.1	PUT	78

5.2.31.3.2	DELETE	79

5.2.31.3.3	GET	79

5.2.31.3.4	PATCH	80

5.2.31A	Resource: SmfSubscriptionInfo	80

5.2.31A.1	Description	80

5.2.31A.2	Resource Definition	81

5.2.31A.3	Resource Standard Methods	81

5.2.31A.3.1	PUT	81

5.2.31A.3.2	DELETE	81

5.2.31A.3.3	GET	82

5.2.31A.3.4	PATCH	82

5.2.32	Resource: ContextData	83

5.2.32.1	Description	83

5.2.32.2	Resource Definition	83

5.2.32.3	Resource Standard Methods	83

5.2.32.3.1	GET	83

5.2.33	Resource: GroupIdentifiers	84

5.2.33.1	Description	84

5.2.33.2	Resource Definition	84

5.2.33.3	Resource Standard Methods	84

5.2.33.3.1	GET	84

5.2.34	Resource: 5GVnGroups	85

5.2.34.1	Description	85

5.2.34.2	Resource Definition	85

5.2.34.3	Resource Standard Methods	85

5.2.34.3.1	GET	85

5.2.35	Resource: Individual5GVnGroup	86

5.2.35.1	Description	86

5.2.35.2	Resource Definition	86

5.2.35.3	Resource Standard Methods	86

5.2.35.3.1	PUT	86

5.2.35.3.2	DELETE	86

5.2.35.3.3	PATCH	87

5.2.35.3.4	GET	88

5.2.36	Resource: LcsPrivacySubscriptionData	88

5.2.36.1	Description	88

5.2.36.2	Resource Definition	88

5.2.36.3	Resource Standard Methods	89

5.2.36.3.1	GET	89

5.2.37	Resource: LcsMobileOriginatedSubscriptionData	89

5.2.37.1	Description	89

5.2.37.2	Resource Definition	89

5.2.37.3	Resource Standard Methods	90

5.2.37.3.1	GET	90

5.2.38	Resource: NiddAuthorizationData	90

5.2.38.1	Description	90

5.2.38.2	Resource Definition	90

5.2.38.3	Resource Standard Methods	91

5.2.38.3.1	GET	91

5.2.39	Resource: CoverageRestrictionData	91

5.2.39.1	Description	91

5.2.39.2	Resource Definition	91

5.2.39.3	Resource Standard Methods	92

5.2.39.3.1	GET	92

5.2.40	Resource: Location	92

5.2.40.1	Description	92

5.2.40.2	Resource Definition	92

5.2.40.3	Resource Standard Methods	92

5.2.40.3.1	GET	92

5.2.41	Resource: V2xSubscriptionData	93

5.2.41.1	Description	93

5.2.41.2	Resource Definition	93

5.2.41.3	Resource Standard Methods	93

5.2.41.3.1	GET	93

5.2.42	Resource: LcsBroadcastAssistanceSubscriptionData	94

5.2.42.1	Description	94

5.2.42.2	Resource Definition	94

5.2.42.3	Resource Standard Methods	94

5.2.42.3.1	GET	94

5.2.43	Resource: 5GVnGroupsInternal	95

5.2.43.1	Description	95

5.2.43.2	Resource Definition	95

5.2.43.3	Resource Standard Methods	95

5.2.43.3.1	GET	95

5.3	Notifications	96

5.3.1	General	96

5.3.2	Data Change Notification	96

5.3.3	Data Removal Notification	97

5.4	Data Model	97

5.4.1	General	97

5.4.2	Structured data types	99

5.4.2.1	Introduction	99

5.4.2.2	Type: AuthenticationSubscription	100

5.4.2.3	Type: OperatorSpecificDataContainer	101

5.4.2.4	Type: SmfRegList	102

5.4.2.5	Type: SubscriptionDataSubscriptions	102

5.4.2.6	Type: DataChangeNotify	103

5.4.2.7	Type: IdentityData	103

5.4.2.8	Type: ProvisionedDataSets	104

5.4.2.9	Type: SorData	104

5.4.2.9A	Type: UpuData	104

5.4.2.9B	Type: NssaiAckData	105

5.4.2.9C	Type: CagAckData	105

5.4.2.10	Void	105

5.4.2.11	Void	105

5.4.2.12	Void	105

5.4.2.13	Void	105

5.4.2.14	Void	105

5.4.2.15	Void	105

5.4.2.16	Void	105

5.4.2.17	Void	105

5.4.2.18	Void	105

5.4.2.19	Type: AmfSubscriptionInfo	106

5.4.2.20	Type: EeProfileData	106

5.4.2.21	Void	106

5.4.2.22	Type: ContextDataSets	107

5.4.2.23	Type: SequenceNumber	108

5.4.2.24	Type: MessageWaitingData	108

5.4.2.25	Type: SmscData	109

5.4.2.26	Type: SmfSubscriptionInfo	109

5.4.2.27	Type: SmfSubscriptionItem	109

5.4.2.28	Type: MtcProvider	109

5.4.3	Simple data types and enumerations	109

5.4.3.1	Introduction	109

5.4.3.2	Simple data types	109

5.4.3.3	Enumeration: AuthMethod	110

5.4.3.4	Enumeration: DataSetName	110

5.4.3.5	Void	110

5.4.3.6	Enumeration: ContextDataSetName	110

5.4.3.7	Enumeration: SqnScheme	111

5.4.3.8	Enumeration: Sign	111

5.4.3.9	Enumeration: UeUpdateStatus	111

5.4.4	Binary data	111

5.5	Error handling	112

5.6	Feature negotiation	112

Annex A (normative):	OpenAPI specification	112

A.1	General	112

A.2	Nudr_DataRepository API for Subscription Data	112

Annex B (informative):	Change history	177

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the usage of the Unified Data Repository services for subscription data. It provides the resource definition and data model for subscription data used over the Nudr Service Based Interface. The protocol definition for Nudr Service Based Interface which is specific to subscription data is also specified in the present document.

The stage 3 protocol definition for Nudr Service Based Interface which is common for subscription data, policy data, structure data and application data for exposure is specified in 3GPP TS 29.504 [2].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 29.504: "5G System; Unified Data Repository Services; Stage 3".

[3]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces Stage 3".

[4]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[5]	3GPP TS 23.003: "Numbering, addressing and identification".

[6]	3GPP TS 29.503: "Unified Data Management Services".

[7]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[8]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[9]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[10]	3GPP TS 33.102: "3G Security; Security architecture".

[11]	3GPP TS 35.206: "An example algorithm set for the 3GPP authentication and key generation functions f1, f1*, f2, f3, f4, f5 and f5*; Document 2: Algorithm specification".

[12]	3GPP TS 35.231: "A second example algorithm set for the 3GPP authentication and key generation functions f1, f1*, f2, f3, f4, f5 and f5*; Document 1: Algorithm specification ".

[13]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[14]	3GPP TR 21.900: "Technical Specification Group working methods".

[15]	3GPP TS 29.509: "Authentication Server Services; Stage 3".

[16]	IETF RFC 3986: "Uniform Resource Identifier (URI): Generic Syntax".

[17]	3GPP TS 29.519: "5G System; Usage of the Unified Data Repository service for Policy Data, Application Data and Structured Data for Exposure; Stage 3".

[18]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS); Stage 2".

[19]	3GPP TS 29.510: "Network Function Repository Services; Stage 3".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

pointer of attribute: The pointer of the attribute identifies the location of the attribute in the JSON object. The pointer of the attribute is a string with one or more segments, each segment is the name of an attribute within that JSON object, different segments are separated with a "/".

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core

AMF	Access and Mobility Management Function

DNN	Data Network Name

GPSI	Generic Public Subscription Identifier

JSON	Javascript Object Notation

SMF	Session Management Function

SMS	Short Message Service

SMSF	SMS Function

SUPI	Subscription Permanent Identifier

UDM	Unified Data Management

UDR	Unified Data Repository

# 4	Overview

The resources, data model and usage defined in the present document are used by the 5GC NFs (e.g. UDM) to access the subscription data via the Unified Data Repository services specified in 3GPP TS 29.504 [2].

# 5	Usage of Nudr_DataRepository Service

## 5.1	Introduction

The following clauses specify the usage of Nudr_DataRepository service for subscription data. The principles specified in 3GPP TS 29.504 [2] are followed unless explicitly specified otherwise in the following clauses.

## 5.2	Resources

### 5.2.1	Overview

# Figure 5.2.1-1: Resource URI sub-level structure for subscription data

# Figure 5.2.1-2: Resource URI sub-level structure for subscription data (cont.)

# Figure 5.2.1-3: Resource URI sub-level structure for subscription data (cont.)

Table 5.2.1-1 provides an overview of the resources and applicable HTTP methods.

# Table 5.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| AuthenticationSubscription | /subscription-data/{ueId}/authentication-data/authentication-subscription | GET | Retrieve a UE's authentication subscription data |
|  |  | PATCH | Update a UE's authentication subscription data Updates shall be limited to the sequenceNumber attribute. Attempts to patch any other attribute shall be rejected by the UDR. |
| AuthenticationSoR | /subscription-data/{ueId}/ue-update-confirmation-data/sor-data | PUT | Store UE's SOR acknowledgement information (SoR-XMAC-IUE) |
|  |  | GET | Retrieve UE's SoR acknowledgement information |
| AuthenticationUPU | /subscription-data/{ueId}/ue-update-confirmation-data/upu-data | PUT | Store UE Parameter Update acknowledgement information (UPU-XMAC-IUE) |
|  |  | GET | Retrieve UE Parameter Update acknowledgement information |
| SubscribedSNSSAIs | /subscription-data/{ueId}/ue-update-confirmation-data/subscribed-snssais | PUT | Store UE-acknowledgement info for change of subscribed S-NSSAIs |
|  |  | GET | Retrieve UE-acknowledgement info for change of subscribed S-NSSAIs |
| SubscribedCAG | /subscription-data/{ueId}/ue-update-confirmation-data/subscribed-cag | PUT | Store UE-acknowledgement info for change of subscribed CAG |
|  |  | GET | Retrieve UE-acknowledgement info for change of subscribed CAG |
| AuthenticationStatus | /subscription-data/{ueId}/authentication-data/authentication-status | PUT | Store a UE's authentication status |
|  |  | GET | Retrieve a UE's authentication status |
|  |  | DELETE | Delete a UE's authentication status |
| IndividualAuthenticationStatus | /subscription-data/{ueId}/authentication-data/authentication-status/{servingNetworkName} | PUT | When the feature "PerUePerSnAuthStatus" is supported, store a UE's Individual authentication status |
|  |  | GET | When the feature "PerUePerSnAuthStatus" is supported, retrieve a UE's Individual authentication status |
|  |  | DELETE | When the feature "PerUePerSnAuthStatus" is supported, delete a UE's Individual authentication status |
| ProvisionedData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data | GET | Retrieve the UE's subscribed Provisioned Data |
| AccessAndMobilitySubscriptionData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/am-data | GET | Retrieve the UE's subscribed Access and Mobility Data |
| SmfSelectionSubscriptionData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/smf-selection-subscription-data | GET | Retrieve the UE's subscribed SMF Selection Data |
| SessionManagementSubscriptionData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/sm-data | GET | Retrieve the UE's subscribed SM Subscription Data |
| ContextData | /subscription-data/{ueId}/context-data | GET | Retrieve the UE's context Data |
| Amf3GppAccessRegistration | /subscription-data/{ueId}/context-data/amf-3gpp-access | PUT | Create and Update the AMF registration for 3GPP access |
|  |  | PATCH | Modify the AMF registration for 3GPP access |
|  |  | GET | Retrieve the AMF registration information for 3GPP access |
| AmfNon3GppAccessRegistration | /subscription-data/{ueId}/context-data/amf-non-3gpp-access | PUT | Update the AMF registration for non 3GPP access |
|  |  | PATCH | Modify the AMF registration for non 3GPP access |
|  |  | GET | Retrieve the AMF registration information for non 3GPP access |
| SmfRegistrations | /subscription-data/{ueId}/context-data/smf-registrations | GET | Retrieve the list of the SMF registrations |
| IndividualSmfRegistration | /subscription-data/{ueId}/context-data /smf-registrations/{pduSessionId} | PUT | Store an individual SMF registration identified by PDU Session Id |
|  |  | DELETE | Delete an individual SMF registration |
|  |  | GET | Retrieve individual SMF registration |
| OperatorSpecificData | /subscription-data/{ueId}/operator-specific-data | GET | retrieve the operator specific subscription data of a UE |
|  |  | PATCH | modify the operator specific subscription data of a UE |
| OperatorDeterminedBarringData | /subscription-data/{ueId}/operator-determined-barring-data | GET | Retrieve the UE's Operator Determined Barring |
| SMSManagementSubscriptionData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/sms-mng-data | GET | Retrieve the UE's subscribed SMS management subscription data. |
| Smsf3GppAccessRegistration | /subscription-data/{ueId}/context-data /smsf-3gpp-access | PUT | Create or Update the SMSF registration |
|  |  | DELETE | Delete the SMSF registration for 3GPP access |
|  |  | GET | Retrieve the SMSF registration information |
| SmsfNon3GppAccessRegistration | /subscription-data/{ueId}/context-data /smsf-non-3gpp-access | PUT | Create or Update the SMSF registration for non 3GPP access |
|  |  | DELETE | Delete the SMSF registration for non 3GPP access |
|  |  | GET | Retrieve the SMSF registration information for non 3GPP access |
| IpSmGwRegistration | /subscription-data/{ueId}/context-data/ip-sm-gw | PUT | Create or Update the IP-SM-GW registration |
|  |  | DELETE | Delete the IP-SM-GW registration |
|  |  | PATCH | Modify the IP-SM-GW registration |
|  |  | GET | Retrieve the IP-SM-GW registration information |
| MessageWaitingData | /subscription-data/{ueId}/context-data/mwd | PUT | Create or Update the SMS Message Waiting Data |
|  |  | DELETE | Delete the SMS Message Waiting Data |
|  |  | PATCH | Modify the SMS Message Waiting Data |
|  |  | GET | Retrieve the SMS Message Waiting Data |
| SdmSubscriptions | /subscription-data/{ueId}/context-data/sdm-subscriptions | GET | Retrieve SDM subscriptions |
|  |  | POST | Create an individual SDM subscription |
| IndividualSdmSubscription | /subscription-data/{ueId}/context-data/sdm-subscriptions/{subsId} | PUT | Update an individual SDM subscription |
|  |  | DELETE | Delete an individual SDM subscription |
|  |  | PATCH | Update an individual SDM Subscription |
|  |  | GET | Retrieve an individual SDM subscription |
| EeSubscriptions | /subscription-data/{ueId}/context-data/ee-subscriptions | GET | Retrieve EE subscriptions |
|  |  | POST | Create an EE subscription |
| IndividualEeSubscription | /subscription-data/{ueId}/context-data/ee-subscriptions/{subsId} | PUT | Update an individual EE subscription |
|  |  | DELETE | Delete an individual EE subscription |
|  |  | PATCH | Update an individual EE subscription |
|  |  | GET | Retrieve an individual EE subscription |
| AmfSubscriptionInfo | /subscription-data/{ueId}/context-data/ee-subscriptions/{subsId}/amf-subscriptions | PUT | Store information related to the Amf-EE-Subscription response |
|  |  | DELETE | Delete the Amf-EE-subscriptions |
|  |  | GET | Retrieve AMF-subscriptions |
|  |  | PATCH | Update AMF-subscriptions |
| SmfSubscriptionInfo | /subscription-data/{ueId}/context-data/ee-subscriptions/{subsId}/smf-subscriptions | PUT | Store information related the a received Smf-EE-Subscription response |
|  |  | DELETE | Delete the Smf-EE-subscriptions |
|  |  | GET | Retrieve SMF-subscriptions |
|  |  | PATCH | Update SMF-subscriptions |
| EeProfileData | /subscription-data/{ueId}/ee-profile-data | GET | Retrieve the UE's subscribed EE profile data. |
| ProvisionedParamenterData | /subscription-data/{ueId}/pp-data | PATCH | Update of provisioned parameters. |
|  |  | GET | Retrieves the UE's provisioned parameters. |
| SMSSubscriptionData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/sms-data | GET | Retrieve the UE's subscribed SMS subscription data. |
| SubscriptionDataSubscriptions | /subscription-data/subs-to-notify | GET | Retrieve existing subscriptions |
|  |  | POST | Create a subscription, i.e. subscribe a node to receive notification for change of data |
|  |  | DELETE | Deletes multiple subscriptions for a given UE. |
| IndividualSubscriptionDataSubscription | /subscription-data/subs-to-notify/{subsId} | DELETE | Delete the subscription identified by {subsId}, i.e. unsubscribe a node to receive notification for change of data |
|  |  | PATCH | Update an individual Subscription to notification |
|  |  | GET | Retrieve an individual Subscription to notification |
| EeGroupSubscriptions | /subscription-data/group-data/{ueGroupId}/ee-subscriptions | GET | Retrieve EE subscriptions for groups of UEs |
|  |  | POST | Create an EE subscription for groups of UEs |
| IndividualEeGroupSubscription | /subscription-data/group-data/{ueGroupId}/ee-subscriptions/{subsId} | PUT | Update an individual EE subscription for a group of UEs |
|  |  | DELETE | Delete an individual EE subscription for a group of UEs |
|  |  | PATCH | Update an individual EE subscription for a group of UEs |
|  |  | GET | Retrieve an individual EE subscription for a group of UEs |
| TraceData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/trace-data | GET | Retrieve the UE's trace configuration data |
| IdentityData | /subscription-data/{ueId}/identity-data | GET | Retrieve identity data that corresponds to the provided ueId |
| SharedData (Collection) | /subscription-data/shared-data | GET | Retrieve shared data |
| GroupIdentifiers | /subscription-data/group-data/group-identifiers | GET | Retrieve group identifiers and the UE identifiers belong to the group identifiers. |
| 5GVnGroups | /subscription-data/group-data/5g-vn-groups | GET | Retrieve 5G VN Groups |
| Individual5GVnGroup | /subscription-data/group-data/5g-vn-groups/{externalGroupId} | PUT | Create a 5G VN Group |
|  |  | PATCH | Update a 5G VN Group |
|  |  | DELETE | Delete a 5G VN Group |
|  |  | GET | Retrieve a 5G VN Group |
| 5GVnGroupsInternal | /subscription-data/group-data/5g-vn-groups/internal | GET | Retrieve 5G VN Group Data based on Internal Group Identifier(s) |
| LcsPrivacySubscriptionData | /subscription-data/{ueId}/lcs-privacy-data | GET | Retrieve the UE's subscribed LCS privacy Subscription Data |
| LcsMobileOriginatedSubscriptionData | /subscription-data/{ueId}/lcs-mo-data | GET | Retrieve the UE's subscribed LCS Mobile Originated Subscription Data |
| NiddAuthorizationData | /subscription-data/{ueId}/nidd-authorization-data | GET | Retrieve the UE's NIDD Authorization Data |
| CoverageRestrictionData | /subscription-data/{ueId}/coverage-restriction-data | GET | Retrieve the UE's subscribed enhanced Coverage Restriction Data |
| Location | /subscription-data/{ueId}/context-data/location | GET | Retrieve the UE's Location Information |
| V2xSubscriptionData | /subscription-data/{ueId}/v2x-data | GET | Retrieve the UE's subscribed V2X Data |
| LcsBroadcastAssistanceSubscriptionData | /subscription-data/{ueId}/{servingPlmnId}/provisioned-data/lcs-bca-data | GET | Retrieve the UE's subscribed LCS Broadcast Assistance subscription data |

### 5.2.2	Resource: AuthenticationSubscription

#### 5.2.2.1	Description

# This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

NOTE 1:	This resource contains security-sensitive attributes, such as the long-term key of the UE (see "encPermanentKey" attribute of the AuthenticationSubscription data type in clause 5.4.2.2). Read/write access can be authorized by means of OAuth2.

NOTE 2:	Although these security-sensitive attributes are stored in an encrypted form in the UDR, operators can consider its storage separately from other types of subscription data. How to achieve this in a secure way is implementation-specific; a description of a typical mechanism can be found in the informative Annex X in 3GPP TS 29.500 [8].

#### 5.2.2.2	Resource Definition

Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/authentication-data/authentication-subscription

This resource shall support the resource URI variables defined in table 5.2.2.2-1.

# Table 5.2.2.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| ueId | Represents the Subscription Identifier of the UE. It may be the SUPI (see 3GPP TS 23.501 [4], clause 5.9.2) of the UE or a pseudonym in SUPI format (e.g. the GLI or GCI of the UE; see 3GPP TS 23.316 [18], clauses 4.7.3 and 4.7.4). Pattern: See pattern of type Supi in 3GPP TS 29.571 [3]. |

#### 5.2.2.3	Resource Standard Methods

##### 5.2.2.3.1	GET

This method shall support the URI query parameters specified in table 5.2.2.3.1-1.

# Table 5.2.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [8] clause 6.6 |

This method shall support the request data structures specified in table 5.2.2.3.1-2 and the response data structures and response codes specified in table 5.2.2.3.1-3.

# Table 5.2.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AuthenticationSubscription | M | 1 | 200 OK | Upon success, a response body containing the AuthenticationSubscription shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.2.3.2	PATCH

This method is used to modify the authentication data of UE in the UDR.

This method shall support the URI query parameters specified in table 5.2.2.3.2-1.

# Table 5.2.2.3.2-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |

This method shall support the request data structures specified in table 5.2.2.3.2-2 and the response data structures and response codes specified in table 5.2.2.3.2-3.

# Table 5.2.2.3.2-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(PatchItem) | M | 1..N | Contains the delta data to the authentication subscription of a UE |

# Table 5.2.2.3.2-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon successful modification there is no body in the response message. (NOTE 2) |
| PatchResult | M | 1 | 200 OK | Upon success, the execution report is returned. (NOTE 2) |
| ProblemDetails | O | 0..1 | 403 Forbidden | If one or more attributes are not allowed to be modified according to e.g. policy or local configuration, then the invalidParams attribute shall contain the JSON pointers of attributes which are not allowed to be modifiedand the cause attribute shall be set to "MODIFICATION_NOT_ALLOWED", see 3GPP TS 29.500 [8] table 5.2.7.2-1. |
| NOTE 1: In addition common data structures as listed in table 5.5-1 are supported. NOTE 2: If all the modification instructions in the PATCH request have been implemented, the UDR shall respond with 204 No Content response; if some of the modification instructions in the PATCH request have been discarded, and the NF service consumer has included in the supported-feature query parameter the "PatchReport" feature number, the UDR shall respond with PatchResult. |  |  |  |  |

### 5.2.3	Resource: AccessAndMobilitySubscriptionData

#### 5.2.3.1	Description

This resource represents the subscribed AccessAndMobilitySubscriptionData for a SUPI for use in a serving PLMN. It is queried by the AMF via the UDM after registering.

# This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

#### 5.2.3.2	Resource Definition

Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/{servingPlmnId}/provisioned-data/am-data

This resource shall support the resource URI variables defined in table 5.2.3.2-1.

# Table 5.2.3.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| ueId | Represents the Subscription Identifier SUPI or GPSI (see 3GPP TS 23.501 [4] clause 5.9.2) pattern: See pattern of type VarUeId 3GPP TS 29.571 [3] |
| servingPlmnId | Represents the Serving PLMN ID (<MCC><MNC>) pattern: "^[0-9]{5,6}$" |

#### 5.2.3.3	Resource Standard Methods

##### 5.2.3.3.1	GET

This method shall support the URI query parameters specified in table 5.2.3.3.1-1.

# Table 5.2.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| fields | array(string) | C | 1..N | When the NF consumer only retrieves a subset of the resource, the " fields " query parameter shall be included. The " fields " query parameter contains the pointers of the attribute(s) to be retrieved. |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [8] clause 6.6 |

This method shall support the request data structures specified in table 5.2.3.3.1-2 and the response data structures and response codes specified in table 5.2.3.3.1-3.

# Table 5.2.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AccessAndMobilitySubscriptionData | M | 1 | 200 OK | Upon success, a response body containing the Access and Mobility Subscription Data shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

### 5.2.4	Resource: SmfSelectionSubscriptionData

#### 5.2.4.1	Description

This resource represents the subscribed SmfSelectionSubscriptionData for a SUPI. It is queried by the UDM triggered  by the AMF after registering.

# This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

#### 5.2.4.2	Resource Definition

Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/{servingPlmnId}/provisioned-data/smf-selection-subscription-data

This resource shall support the resource URI variables defined in table 5.2.4.2-1.

# Table 5.2.4.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| ueId | Represents the Subscription Identifier SUPI or GPSI (see 3GPP TS 23.501 [4] clause 5.9.2) pattern: See pattern of type VarUeId 3GPP TS 29.571 [3] |
| servingPlmnId | Represents the Serving PLMN ID (<MCC><MNC>) pattern: "^[0-9]{5,6}$" |

#### 5.2.4.3	Resource Standard Methods

##### 5.2.4.3.1	GET

This method shall support the URI query parameters specified in table 5.2.4.3.1-1.

# Table 5.2.4.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| fields | array(string) | C | 1..N | When the NF consumer only retrieves a subset of the resource, the " fields " query parameter shall be included. The " fields " query parameter contains the pointers of the attribute(s) to be retrieved. |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [8] clause 6.6 |

This method shall support the request data structures specified in table 5.2.4.3.1-2 and the response data structures and response codes specified in table 5.2.4.3.1-3.

# Table 5.2.4.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.4.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SmfSelectionSubscriptionData | M | 1 | 200 OK | Upon success, a response body containing the SMF Selection Subscription Data for the user shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

### 5.2.5	Resource: SessionManagementSubscriptionData

#### 5.2.5.1	Description

This resource represents the subscribed SessionManagementSubscriptionData for a SUPI. It is queried by the UDM triggered by the SMF during session setup, using one or both of query parameters representing the selected network slice and the DNN.

# This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

#### 5.2.5.2	Resource Definition

Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/{servingPlmnId}/provisioned-data/sm-data

This resource shall support the resource URI variables defined in table 5.2.5.2-1.

# Table 5.2.5.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| ueId | Represents the Subscription Identifier SUPI or GPSI (see 3GPP TS 23.501 [4] clause 5.9.2) pattern: See pattern of type VarUeId 3GPP TS 29.571 [3] |
| servingPlmnId | Represents the Serving PLMN ID (<MCC><MNC>) pattern: "^[0-9]{5,6}$" |

#### 5.2.5.3	Resource Standard Methods

##### 5.2.5.3.1	GET

This method shall support the URI query parameters specified in table 5.2.5.3.1-1.

# Table 5.2.5.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| single-nssai | Snssai | O | 0..1 | When present without Slice Differentiator (sd), all slices identified by the given Slice/Service Type (sst) and any sd value (if any) shall be considered matching the query parameter. |
| dnn | Dnn | O | 0..1 |  |
| fields | array(string) | C | 1..N | When the NF consumer only retrieves a subset of the resource, the " fields " query parameter shall be included. The " fields " query parameter contains the pointers of the attribute(s) to be retrieved. |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [8] clause 6.6 |

If "single-nssai" is not included, and "dnn" is not included, UDR shall return all DNN configurations for all network slice(s).

If "single-nssai" is included, and "dnn" is not included, UDR shall return all DNN configurations for the requested network slice identified by "single-nssai".

If "single-nssai" is not included, and "dnn" is included, UDR shall return all DNN configurations identified by "dnn" for all network slices where such DNN is available.

If "single-nssai" is included, and "dnn" is included, UDR shall return the DNN configuration identified by "dnn", if such DNN is available in the network slice identified by "single-nssai".

This method shall support the request data structures specified in table 5.2.5.3.1-2 and the response data structures and response codes specified in table 5.2.5.3.1-3.

# Table 5.2.5.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.5.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(SessionManagementSubscriptionData) | M | 1..N | 200 OK | Upon success, a response body containing the Session Management Subscription data shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

### 5.2.6	Resource: Amf3GppAccessRegistration

#### 5.2.6.1	Description

This resource is used to represent AMF registrations to be stored in the UDR by the UDM.

# This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

#### 5.2.6.2	Resource Definition

Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/context-data/amf-3gpp-access

This resource shall support the resource URI variables defined in table 5.2.6.2-1.

# Table 5.2.6.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| ueId | Represents the Subscription Identifier SUPI or GPSI (see 3GPP TS 23.501 [4] clause 5.9.2) pattern: See pattern of type VarUeId 3GPP TS 29.571 [3] |

#### 5.2.6.3	Resource Standard Methods

##### 5.2.6.3.1	PUT

This method shall support the URI query parameters specified in table 5.2.6.3.1-1.

# Table 5.2.6.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.6.3.1-2 and the response data structures and response codes specified in table 5.2.6.3.1-3.

# Table 5.2.6.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| Amf3GppAccessRegistration | M | 1 | The AMF registration for 3GPP access is replaced with the received information. |

# Table 5.2.6.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned |
| Amf3GppAccessRegistration | M | 1 | 201 Created | Upon success, a response body containing a representation of the created Individual Amf3GppAccessRegistration resource shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.6.3.2	PATCH

This method is used to modify AMF registration data for 3GPP access in the UDR.

This method shall support the URI query parameters specified in table 5.2.6.3.2-1.

# Table 5.2.6.3.2-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |

This method shall support the request data structures specified in table 5.2.6.3.2-2 and the response data structures and response codes specified in table 5.2.6.3.2-3.

# Table 5.2.6.3.2-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(PatchItem) | M | 1..N | Contains the delta data to the AMF registration data for 3GPP access |

# Table 5.2.6.3.2-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon successful modification there is no body in the response message. (NOTE 2) |
| PatchResult | M | 1 | 200 OK | Upon success, the execution report is returned. (NOTE 2) |
| ProblemDetails | O | 0..1 | 403 Forbidden | If one or more attributes are not allowed to be modified according to e.g. policy or local configuration, thenthe invalidParams attribute shall contain the JSON pointers of attributes which are not allowed to be modified and the cause attribute shall be set to "MODIFICATION_NOT_ALLOWED", see 3GPP TS 29.500 [8] table 5.2.7.2-1. |
| NOTE 1: In addition common data structures as listed in table 5.5-1 are supported. NOTE 2: If all the modification instructions in the PATCH request have been implemented, the UDR shall respond with 204 No Content response; if some of the modification instructions in the PATCH request have been discarded, and the NF service consumer has included in the supported-feature query parameter the "PatchReport" feature number, the UDR shall respond with PatchResult. |  |  |  |  |

##### 5.2.6.3.3	GET

# This method shall support the URI query parameters specified in table 5.2.6.3.1-1.

# Table 5.2.6.3.3-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| fields | array(string) | C | 1..N | When the NF consumer only retrieves a subset of the resource, the " fields " query parameter shall be included. The " fields " query parameter contains the pointers of the attribute(s) to be retrieved. |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [8] clause 6.6 |

This method shall support the request data structures specified in table 5.2.6.3.3-2 and the response data structures and response codes specified in table 5.2.6.3.3-3.

# Table 5.2.6.3.3-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.6.3.3-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| Amf3GppAccessRegistration | M | 1 | 200 OK | Upon success, a response body containing the Amf3GppAccessRegistration shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

### 5.2.7	Resource: AmfNon3GppAccessRegistration

#### 5.2.7.1	Description

This resource is used to represent AMF registrations to be stored in the UDR.

# This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

#### 5.2.7.2	Resource Definition

Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/context-data/amf-non-3gpp-access

This resource shall support the resource URI variables defined in table 5.2.7.2-1.

# Table 5.2.7.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| ueId | Represents the Subscription Identifier SUPI or GPSI (see 3GPP TS 23.501 [4] clause 5.9.2) pattern: See pattern of type VarUeId 3GPP TS 29.571 [3] |

#### 5.2.7.3	Resource Standard Methods

##### 5.2.7.3.1	PUT

This method shall support the URI query parameters specified in table 5.2.7.3.1-1.

# Table 5.2.7.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.7.3.1-2 and the response data structures and response codes specified in table 5.2.7.3.1-3.

# Table 5.2.7.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AmfNon3GppAccessRegistration | M | 1 | The AMF registration for non 3GPP access is replaced with the received information. |

# Table 5.2.7.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned |
| AmfNon3GppAccessRegistration | M | 1 | 201 Created | Upon success, a response body containing a representation of the created Individual AmfNon3GppAccessRegistration resource shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.7.3.2	PATCH

This method is used to modify AMF registration data for non-3GPP access in the UDR.

This method shall support the URI query parameters specified in table 5.2.7.3.2-1.

# Table 5.2.7.3.2-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |

This method shall support the request data structures specified in table 5.2.7.3.2-2 and the response data structures and response codes specified in table 5.2.7.3.2-3.

# Table 5.2.7.3.2-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(PatchItem) | M | 1..N | Contains the delta data to the AMF registration data for non-3GPP access |

# Table 5.2.7.3.2-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon successful modification there is no body in the response message. (NOTE 2) |
| PatchResult | M | 1 | 200 OK | Upon success, the execution report is returned. (NOTE 2) |
| ProblemDetails | O | 0..1 | 403 Forbidden | If one or more attributes are not allowed to be modified according to e.g. policy or local configuration, thenthe invalidParams attribute shall contain the JSON pointers of attributes which are not allowed to be modified, andthe cause attribute shall be set to "MODIFICATION_NOT_ALLOWED", see 3GPP TS 29.500 [8] table 5.2.7.2-1. |
| NOTE 1: In addition common data structures as listed in table 5.5-1 are supported. NOTE 2: If all the modification instructions in the PATCH request have been implemented, the UDR shall respond with 204 No Content response; if some of the modification instructions in the PATCH request have been discarded, and the NF service consumer has included in the supported-feature query parameter the "PatchReport" feature number, the UDR shall respond with PatchResult. |  |  |  |  |

##### 5.2.7.3.3	GET

This method shall support the URI query parameters specified in table 5.2.7.3.1-1.

# Table 5.2.7.3.3-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| fields | array(string) | C | 1..N | When the NF consumer only retrieves a subset of the resource, the " fields " query parameter shall be included. The " fields " query parameter contains the pointers of the attribute(s) to be retrieved. |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [8] clause 6.6 |

This method shall support the request data structures specified in table 5.2.7.3.3-2 and the response data structures and response codes specified in table 5.2.7.3.3-3.

# Table 5.2.7.3.3-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.7.3.3-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AmfNon3GppAccessRegistration | M | 1 | 200 OK | Upon success, a response body containing the Amf3GppAccessRegistration shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

### 5.2.8	Resource: SmfRegistrations

#### 5.2.8.1	Description

This resource is used to represent SMF registrations to be stored in the UDR.

# This resource is modelled with the Store resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

#### 5.2.8.2	Resource Definition

Resource URI: {apiRoot}/nudr-dr/<apiVersion>/ subscription-data/{ueId}/context-data/smf-registrations

This resource shall support the resource URI variables defined in table 6.2.3.4.2-1.

# Table 5.2.8.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.4.1 |
| ueId | Represents the Subscription Identifier SUPI or GPSI (see 3GPP TS 23.501 [4] clause 5.9.2) pattern: See pattern of type VarUeId 3GPP TS 29.571 [3] |

#### 5.2.8.3	Resource Standard Methods

##### 5.2.8.3.1	GET

This method shall support the URI query parameters specified in table 5.2.8.3.1-1.

# Table 5.2.8.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [8] clause 6.6 |

This method shall support the request data structures specified in table 5.2.8.3.1-2 and the response data structures and response codes specified in table 5.2.8.3.1-3.

# Table 5.2.8.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.8.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SmfRegList | M | 1 | 200 OK | Upon success, a response body containing the list of SMF registrations of the relevant UE shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

### 5.2.9	Resource: IndividualSmfRegistration

#### 5.2.9.1	Description

This resource is used to represent individual PDU session SMF registrations to be stored in the UDR.

# This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

#### 5.2.9.2	Resource Definition

Resource URI: {apiRoot}/nudr-dr/<apiVersion>/{ueId}/context-data/smf-registrations/{pduSessionId}

This resource shall support the resource URI variables defined in table 5.2.9.2-1.

# Table 5.2.9.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.4.1 |
| ueId | Represents the Subscription Identifier SUPI or GPSI (see 3GPP TS 23.501 [4] clause 5.9.2) pattern: See pattern of type VarUeId 3GPP TS 29.571 [3] |
| pduSessionId | The pduSessionId identifies an individual SMF registration. |

#### 5.2.9.3	Resource Standard Methods

##### 5.2.9.3.1	PUT

This method shall support the URI query parameters specified in table 5.2.9.3.1-1.

# Table 5.2.9.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.9.3.1-2 and the response data structures and response codes specified in table 5.2.9.3.1-3.

# Table 5.2.9.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SmfRegistration | M | 1 | The registration that is to be created |

# Table 5.2.9.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SmfRegistration | M | 1 | 201 Created | Upon success, a response body containing a representation of the created Individual SmfRegistration resource shall be returned. |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.9.3.2	DELETE

This method shall support the URI query parameters specified in table 5.2.9.3.2-1.

# Table 5.2.9.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.9.3.2-2 and the response data structures and response codes specified in table 5.2.9.3.2-3.

# Table 5.2.9.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.9.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.9.3.3	GET

This method shall support the URI query parameters specified in table 5.2.9.3.3-1.

# Table 5.2.9.3.3-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| fields | array(string) | C | 1..N | When the NF consumer only retrieves a subset of the resource, the " fields " query parameter shall be included. The " fields " query parameter contains the pointers of the attribute(s) to be retrieved. |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [8] clause 6.6 |

This method shall support the request data structures specified in table 5.2.9.3.3-2 and the response data structures and response codes specified in table 5.2.9.3.3-3.

# Table 5.2.9.3.3-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.9.3.3-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SmfRegistration | M | 1 | 200 OK | The SMF registration that is to be retrieved |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

### 5.2.10	Resource: OperatorSpecificData

#### 5.2.10.1	Description

This resource represents the UE's operator specific data.

This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

#### 5.2.10.2	Resource Definition

Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/operator-specific-data

This resource shall support the resource URI variables defined in table 5.2.10.2-1.

# Table 5.2.10.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| ueId | Represents the Subscription Identifier SUPI or GPSI (see 3GPP TS 23.501 [4] clause 5.9.2) pattern: See pattern of type VarUeId 3GPP TS 29.571 [3] |
| NOTE: The content of ueId SUPI or GPSI depends on the service. |  |

#### 5.2.10.3	Resource Standard Methods

##### 5.2.10.3.1	GET

This method is used to retrieve operator specific data from the UDR.

This method shall support the URI query parameters specified in table 5.2.10.3.1-1.

# Table 5.2.10.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| fields | array(string) | C | 1..N | When the NF consumer only retrieves a subset of the resource, the " fields " query parameter shall be included. The " fields " query parameter contains the pointers of the attribute(s) to be retrieved. |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [8] clause 6.6 |

This method shall support the request data structures specified in table 5.2.10.3.1-2 and the response data structures and response codes specified in table 5.2.10.3.1-3.

# Table 5.2.10.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.10.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| map(OperatorSpecificDataContainer) | M | 1..N | 200 OK | Upon success, a response body containing a map shall be returned. The key of the map is operator specific data element name and the value is the operator specific data of the UE. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.10.3.2	PATCH

This method is used to modify operator specific data in the UDR.

This method shall support the URI query parameters specified in table 5.2.10.3.2-1.

# Table 5.2.10.3.2-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |

This method shall support the request data structures specified in table 5.2.10.3.2-2 and the response data structures and response codes specified in table 5.2.10.3.2-3.

# Table 5.2.10.3.2-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(PatchItem) | M | 1..N | Contains the delta data to the operator specific data |

# Table 5.2.10.3.2-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon successful modification there is no body in the response message. (NOTE 2) |
| PatchResult | M | 1 | 200 OK | Upon success, the execution report is returned. (NOTE 2) |
| ProblemDetails | O | 0..1 | 403 Forbidden | If one or more attributes are not allowed to be modified according to e.g. policy or local configuration, thenthe invalidParams attribute shall contain the JSON pointers of attributes which are not allowed to be modified, and the cause attribute shall be set to "MODIFICATION_NOT_ALLOWED", see 3GPP TS 29.500 [8] table 5.2.7.2-1. |
| NOTE 1: In addition common data structures as listed in table 5.5-1 are supported. NOTE 2: If all the modification instructions in the PATCH request have been implemented, the UDR shall respond with 204 No Content response; if some of the modification instructions in the PATCH request have been discarded, and the NF service consumer has included in the supported-feature query parameter the "PatchReport" feature number, the UDR shall respond with PatchResult. |  |  |  |  |

### 5.2.11	Resource: Smsf3GppAccessRegistration

#### 5.2.11.1	Description

# This resource represents registered SMSF for 3GPP access.

# This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).-

#### 5.2.11.2	Resource Definition

# Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/context-data/smsf-3gpp-access

# This resource shall support the resource URI variables defined in table 5.2.11.2-1.

# Table 5.2.11.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| ueId | Represents the Subscription Identifier SUPI or GPSI (see 3GPP TS 23.501 [4] clause 5.9.2) pattern: See pattern of type VarUeId 3GPP TS 29.571 [3] |

#### 5.2.11.3	Resource Standard Methods

##### 5.2.11.3.1	PUT

# This method shall support the URI query parameters specified in table 5.2.11.3.1-1.

# Table 5.2.11.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.11.3.1-2 and the response data structures and response codes specified in table 5.2.11.3.1-3.

# Table 5.2.11.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SmsfRegistration | M | 1 | The SMSF registration for 3GPP access is created or updated with the received information. |

# Table 5.2.11.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned |
| SmsfRegistration | M | 1 | 201 Created | Upon success, a response body containing a representation of the created Individual SmsfRegistration for 3GPP access resource shall be returned. |
| SmsfRegistration | M | 1 | 200 OK | Upon success, a response body containing a representation of the updated Individual SmsfRegistration for 3GPP access resource shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.11.3.2	DELETE

# This method shall support the URI query parameters specified in table 5.2.11.3.2-1.

# Table 5.2.11.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.11.3.2-2 and the response data structures and response codes specified in table 5.2.11.3.2-3.

# Table 5.2.11.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  | The request body shall be empty. |

# Table 5.2.11.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.11.3.4	GET

# This method shall support the URI query parameters specified in table 5.2.11.3.4-1.

# Table 5.2.11.3.4-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| fields | array(string) | C | 1..N | When the NF consumer only retrieves a subset of the resource, the " fields " query parameter shall be included. The " fields " query parameter contains the pointers of the attribute(s) to be retrieved. |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [8] clause 6.6 |

This method shall support the request data structures specified in table 5.2.11.3.4-2 and the response data structures and response codes specified in table 5.2.11.3.4-3.

# Table 5.2.11.3.4-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.11.3.4-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SmsfRegistration | M | 1 | 200 OK | Upon success, a response body containing the SMSF registration for 3GPP access Data shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

### 5.2.12	Resource: SmsfNon3GppAccessRegistration

#### 5.2.12.1	Description

# This resource represents registered SMSF for Non 3GPP access.

# This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

#### 5.2.12.2	Resource Definition

# Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/context-data/smsf-non-3gpp-access

# This resource shall support the resource URI variables defined in table 5.2.12.2-1.

# Table 5.2.12.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| ueId | Represents the Subscription Identifier SUPI or GPSI (see 3GPP TS 23.501 [4] clause 5.9.2) pattern: See pattern of type VarUeId 3GPP TS 29.571 [3] |

#### 5.2.12.3	Resource Standard Methods

##### 5.2.12.3.1	PUT

# This method shall support the URI query parameters specified in table 5.2.12.3.1-1.

# Table 5.2.12.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.12.3.1-2 and the response data structures and response codes specified in table 5.2.12.3.1-3.

# Table 5.2.12.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SmsfRegistration | M | 1 | The SMSF registration for non 3GPP access is created or updated with the received information. |

# Table 5.2.12.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned |
| SmsfRegistration | M | 1 | 201 Created | Upon success, a response body containing a representation of the created Individual SmsfRegistration for non 3GPP access resource shall be returned. |
| SmsfRegistration | M | 1 | 200 OK | Upon success, a response body containing a representation of the updated Individual SmsfRegistration for non 3GPP access resource shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.12.3.2	DELETE

# This method shall support the URI query parameters specified in table 5.2.12.3.2-1.

# Table 5.2.12.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.12.3.2-2 and the response data structures and response codes specified in table 5.2.12.3.2-3.

# Table 5.2.12.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  | The request body shall be empty. |

# Table 5.2.12.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.12.3.4	GET

# This method shall support the URI query parameters specified in table 5.2.12.3.4-1.

# Table 5.2.12.3.4-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| fields | array(string) | C | 1..N | When the NF consumer only retrieves a subset of the resource, the " fields " query parameter shall be included. The " fields " query parameter contains the pointers of the attribute(s) to be retrieved. |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [8] clause 6.6 |

This method shall support the request data structures specified in table 5.2.12.3.4-2 and the response data structures and response codes specified in table 5.2.12.3.4-3.

# Table 5.2.12.3.4-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.12.3.4-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SmsfRegistration | M | 1 | 200 OK | Upon success, a response body containing the SMSF registration for non 3GPP access Data shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

### 5.2.12A	Resource: IpSmGwRegistration

#### 5.2.12A.1	Description

# This resource represents the registered IP-SM-GW.

# This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

#### 5.2.12A.2	Resource Definition

# Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/context-data/ip-sm-gw

# This resource shall support the resource URI variables defined in table 5.2.12A.2-1.

# Table 5.2.12A.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| ueId | Represents the Subscription Identifier SUPI or GPSI (see 3GPP TS 23.501 [4] clause 5.9.2) pattern: See pattern of type VarUeId 3GPP TS 29.571 [3] |

#### 5.2.12A.3	Resource Standard Methods

##### 5.2.12A.3.1	PUT

# This method shall support the URI query parameters specified in table 5.2.12A.3.1-1.

# Table 5.2.12A.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.12A.3.1-2 and the response data structures and response codes specified in table 5.2.12A.3.1-3.

# Table 5.2.12A.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| IpSmGwRegistration | M | 1 | The IP-SM-GW registration is created or updated with the received information. |

# Table 5.2.12A.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned |
| NOTE: In addition, common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.12A.3.2	DELETE

# This method shall support the URI query parameters specified in table 5.2.12A.3.2-1.

# Table 5.2.12A.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.12A.3.2-2 and the response data structures and response codes specified in table 5.2.12A.3.2-3.

# Table 5.2.12A.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  | The request body shall be empty. |

# Table 5.2.12A.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| NOTE: In addition, common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.12A.3.3	PATCH

This method shall support the URI query parameters specified in table 5.2.12A.3.3-1.

# Table 5.2.12A.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |

This method shall support the request data structures specified in table 5.2.12A.3.3-2 and the response data structures and response codes specified in table 5.2.12A.3.3-3.

# Table 5.2.12A.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(PatchItem) | M | 1..N | Contains the delta data to the authentication subscription of a UE |

# Table 5.2.12A.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon successful modification there is no body in the response message. (NOTE 2) |
| PatchResult | M | 1 | 200 OK | Upon success, the execution report is returned. (NOTE 2) |
| ProblemDetails | O | 0..1 | 403 Forbidden | One or more attributes are not allowed to be modified according to e.g. policy or local configuration. The invalidParams attribute shall contain the JSON pointers of attributes which are not allowed to be modified. The cause attribute may be used to convey any of the following errors: - "MODIFICATION_NOT_ALLOWED", see 3GPP TS 29.500 [8] table 5.2.7.2-1. |
| NOTE 1: In addition, common data structures as listed in table 5.5-1 are supported. NOTE 2: If all the modification instructions in the PATCH request have been implemented, the UDR shall respond with 204 No Content response; if some of the modification instructions in the PATCH request have been discarded, and the NF service consumer has included in the supported-feature query parameter the "PatchReport" feature number, the UDR shall respond with PatchResult. |  |  |  |  |

##### 5.2.12A.3.4	GET

# This method shall support the URI query parameters specified in table 5.2.12A.3.4-1.

# Table 5.2.12A.3.4-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| fields | array(string) | C | 1..N | When the NF consumer only retrieves a subset of the resource, the "fields" query parameter shall be included. The "fields" query parameter contains the pointers of the attribute(s) to be retrieved. |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [8] clause 6.6 |

This method shall support the request data structures specified in table 5.2.12A.3.4-2 and the response data structures and response codes specified in table 5.2.12A.3.4-3.

# Table 5.2.12A.3.4-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.12A.3.4-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| IpSmGwRegistration | M | 1 | 200 OK | Upon success, a response body containing the IP-SM-GW Registration Data shall be returned. |
| NOTE: In addition, common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

### 5.2.12B	Resource: MessageWaitingData

#### 5.2.12B.1	Description

# This resource represents the Message Waiting Data list for a given UE.

# This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

#### 5.2.12B.2	Resource Definition

# Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/context-data/mwd

# This resource shall support the resource URI variables defined in table 5.2.12B.2-1.

# Table 5.2.12B.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| ueId | Represents the Subscription Identifier SUPI or GPSI (see 3GPP TS 23.501 [4] clause 5.9.2) pattern: See pattern of type VarUeId 3GPP TS 29.571 [3] |

#### 5.2.12B.3	Resource Standard Methods

##### 5.2.12B.3.1	PUT

# This method shall support the URI query parameters specified in table 5.2.12B.3.1-1.

# Table 5.2.12B.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.12B.3.1-2 and the response data structures and response codes specified in table 5.2.12B.3.1-3.

# Table 5.2.12B.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MessageWaitingData | M | 1 | The Message Waiting Data list is created or updated with the received information. |

# Table 5.2.12B.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned |
| NOTE: In addition, common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.12B.3.2	DELETE

# This method shall support the URI query parameters specified in table 5.2.12B.3.2-1.

# Table 5.2.12B.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.12B.3.2-2 and the response data structures and response codes specified in table 5.2.12B.3.2-3.

# Table 5.2.12B.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  | The request body shall be empty. |

# Table 5.2.12B.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| NOTE: In addition, common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.12B.3.3	PATCH

This method shall support the URI query parameters specified in table 5.2.12B.3.3-1.

# Table 5.2.12B.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |

This method shall support the request data structures specified in table 5.2.12B.3.3-2 and the response data structures and response codes specified in table 5.2.12B.3.3-3.

# Table 5.2.12B.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(PatchItem) | M | 1..N | Contains the delta data to the authentication subscription of a UE |

# Table 5.2.12B.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon successful modification there is no body in the response message. (NOTE 2) |
| PatchResult | M | 1 | 200 OK | Upon success, the execution report is returned. (NOTE 2) |
| ProblemDetails | O | 0..1 | 403 Forbidden | One or more attributes are not allowed to be modified according to e.g. policy or local configuration. The invalidParams attribute shall contain the JSON pointers of attributes which are not allowed to be modified. The cause attribute may be used to convey any of the following errors: - "MODIFICATION_NOT_ALLOWED", see 3GPP TS 29.500 [8] table 5.2.7.2-1. |
| NOTE 1: In addition, common data structures as listed in table 5.5-1 are supported. NOTE 2: If all the modification instructions in the PATCH request have been implemented, the UDR shall respond with 204 No Content response; if some of the modification instructions in the PATCH request have been discarded, and the NF service consumer has included in the supported-feature query parameter the "PatchReport" feature number, the UDR shall respond with PatchResult. |  |  |  |  |

##### 5.2.12B.3.4	GET

# This method shall support the URI query parameters specified in table 5.2.12B.3.4-1.

# Table 5.2.12B.3.4-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| fields | array(string) | C | 1..N | When the NF consumer only retrieves a subset of the resource, the "fields" query parameter shall be included. The "fields" query parameter contains the pointers of the attribute(s) to be retrieved. |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [8] clause 6.6 |

This method shall support the request data structures specified in table 5.2.12B.3.4-2 and the response data structures and response codes specified in table 5.2.12B.3.4-3.

# Table 5.2.12B.3.4-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.12B.3.4-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MessageWaitingData | M | 1 | 200 OK | Upon success, a response body containing the Message Waiting Data shall be returned. |
| NOTE: In addition, common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

### 5.2.13	Resource: SMSManagementSubscriptionData

#### 5.2.13.1	Description

This resource represents the subscribed SMSManagementSubscriptionData for a SUPI for use in a serving PLMN. It is queried by the SMSF via the UDM after registering.

# This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

#### 5.2.13.2	Resource Definition

# Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/{servingPlmnId}/provisioned-data/sms-mng-data

# This resource shall support the resource URI variables defined in table 5.2.13.2-1.

# Table 5.2.13.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| ueId | Represents the Subscription Identifier SUPI or GPSI (see 3GPP TS 23.501 [4] clause 5.9.2) pattern: See pattern of type VarUeId 3GPP TS 29.571 [3] |
| servingPlmnId | Represents the Serving PLMN ID (<MCC><MNC>) pattern: "^[0-9]{5,6}$" |

#### 5.2.13.3	Resource Standard Methods

##### 5.2.13.3.1	GET

# This method shall support the URI query parameters specified in table 5.2.13.3.1-1.

# Table 5.2.13.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [8] clause 6.6 |

This method shall support the request data structures specified in table 5.2.13.3.1-2 and the response data structures and response codes specified in table 5.2.13.3.1-3.

# Table 5.2.13.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.13.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SmsManagementSubscriptionData | M | 1 | 200 OK | Upon success, a response body containing the SMS Management Subscription Data shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

### 5.2.14	Resource: ProvisionedParamenterData

#### 5.2.14.1	Description

This resource represents the subscribed ProvisionedParameterData for a GPSI. It is triggered by the AF/NEF via the UDM to update provisioned data stored for the subscriber.

# This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

#### 5.2.14.2	Resource Definition

# Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/pp-data

# This resource shall support the resource URI variables defined in table 5.2.14.2-1.

# Table 5.2.14.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| ueId | Represents the Subscription Identifier SUPI or GPSI (see 3GPP TS 23.501 [4] clause 5.9.2) pattern: See pattern of type VarUeId 3GPP TS 29.571 [3] |

#### 5.2.14.3	Resource Standard Methods

##### 5.2.14.3.1	GET

This method retrieves the provisioned data of a given UE.

This method shall support the URI query parameters specified in table 5.2.14.3.1-1.

# Table 5.2.14.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [8] clause 6.6 |

This method shall support the request data structures specified in table 5.2.14.3.1-2 and the response data structures and response codes specified in table 5.2.14.3.1-3.

# Table 5.2.14.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.14.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| PpData | M | 1 | 200 OK | Upon success, a response body is returned containing the provisioned data of the GPSI. |

##### 5.2.14.3.2	PATCH

This method is used to modify the provisioned parameter data in the UDR.

This method shall support the URI query parameters specified in table 5.2.14.3.2-1.

# Table 5.2.14.3.2-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |

This method shall support the request data structures specified in table 5.2.14.3.2-2 and the response data structures and response codes specified in table 5.2.14.3.2-3.

# Table 5.2.14.3.2-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(PatchItem) | M | 1..N | Contains the delta data to the provisioned parameter data |

# Table 5.2.14.3.2-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon successful modification there is no body in the response message. (NOTE 2) |
| PatchResult | M | 1 | 200 OK | Upon success, the execution report is returned. (NOTE 2) |
| ProblemDetails | O | 0..1 | 403 Forbidden | If one or more attributes are not allowed to be modified according to e.g. policy or local configuration, then the invalidParams attribute shall contain the JSON pointers of attributes which are not allowed to be modified, and the cause attribute shall be set to "MODIFICATION_NOT_ALLOWED", see 3GPP TS 29.500 [8] table 5.2.7.2-1. |
| NOTE 1: In addition common data structures as listed in table 5.5-1 are supported. NOTE 2: If all the modification instructions in the PATCH request have been implemented, the UDR shall respond with 204 No Content response; if some of the modification instructions in the PATCH request have been discarded, and the NF service consumer has included in the supported-feature query parameter the "PatchReport" feature number, the UDR shall respond with PatchResult. |  |  |  |  |

### 5.2.15	Resource: SMSSubscriptionData

#### 5.2.15.1	Description

This resource represents the subscribed SMS Subscription Data for a SUPI for use in a serving PLMN. It is queried by the AMF via the UDM after registering.

This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

#### 5.2.15.2	Resource Definition

Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/{serving-plmn-id}/provisioned-data/sms-data

This resource shall support the resource URI variables defined in table 5.2.15.2-1.

# Table 5.2.15.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| Ue-id | Represents the Subscription Identifier SUPI or GPSI (see 3GPP TS 23.501 [4] clause 5.9.2) pattern: See pattern of type VarUeId 3GPP TS 29.571 [3] |
| serving-plmn-id | Represents the Serving PLMN ID (<MCC><MNC>) pattern: "^[0-9]{5,6}$" |

#### 5.2.15.3	Resource Standard Methods

##### 5.2.15.3.1	GET

This method shall support the URI query parameters specified in table 5.2.15.3.1-1.

# Table 5.2.15.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [8] clause 6.6 |

This method shall support the request data structures specified in table 5.2.15.3.1-2 and the response data structures and response codes specified in table 5.2.15.3.1-3.

# Table 5.2.15.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.15.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SmsSubscriptionData | M | 1 | 200 OK | Upon success, a response body containing the SMS Management Subscription Data shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

### 5.2.16	Resource: SdmSubscriptions

#### 5.2.16.1	Description

This resource represents the collection of SDM Subscriptions for a UE.

# This resource is modelled with the Collection resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

#### 5.2.16.2	Resource Definition

# Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/context-data/sdm-subscriptions

# This resource shall support the resource URI variables defined in table 5.2.16.2-1.

# Table 5.2.16.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| ueId | Represents the Subscription Identifier SUPI or GPSI (see 3GPP TS 23.501 [4] clause 5.9.2) pattern: See pattern of type VarUeId 3GPP TS 29.571 [3] |

#### 5.2.16.3	Resource Standard Methods

##### 5.2.16.3.1	GET

# This method shall support the URI query parameters specified in table 5.2.16.3.1-1.

# Table 5.2.16.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [8] clause 6.6 |

This method shall support the request data structures specified in table 5.2.16.3.1-2 and the response data structures and response codes specified in table 5.2.16.3.1-3.

# Table 5.2.16.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.16.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(SdmSubscription) | M | 1..N | 200 OK | Upon success, a response body containing the individual SDM subscriptions shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.16.3.2	POST

This method shall support the URI query parameters specified in table 5.2.16.3.2-1.

# Table 5.2.16.3.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.16.3.1-2 and the response data structures and response codes specified in table 5.2.16.3.1-3.

# Table 5.2.16.3.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SdmSubscription | M | 1 | Identifies the subscriber data subscription to be stored in the UDR. |

# Table 5.2.16.3.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SdmSubscription | M | 1 | 201 Created | Upon success, a response body containing a representation of the UeId's Individual subscription resource shall be returned. The HTTP response shall include a "Location" HTTP header that contains the resource URI of the created resource. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

### 5.2.17	Resource: IndividualSdmSubscription

#### 5.2.17.1	Description

This resource represents an individual SDM Subscription for a UE.

# This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

#### 5.2.17.2	Resource Definition

# Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/context-data/sdm-subscriptions/{subsId}

# This resource shall support the resource URI variables defined in table 5.2.17.2-1.

# Table 5.2.17.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| ueId | Represents the Subscription Identifier SUPI or GPSI (see 3GPP TS 23.501 [4] clause 5.9.2) pattern: See pattern of type VarUeId 3GPP TS 29.571 [3] |
| subsId | Identifier of the subscription (as allocated by the UDR) |

#### 5.2.17.3	Resource Standard Methods

##### 5.2.17.3.1	PUT

# This method shall support the URI query parameters specified in table 5.2.17.3.1-1.

# Table 5.2.17.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.17.3.1-2 and the response data structures and response codes specified in table 5.2.17.3.1-3.

# Table 5.2.17.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SdmSubscription | M | 1 | The individual SDM subscription that is to be stored |

# Table 5.2.17.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| ProblemDetails | O | 0..1 | 404 Not Found | If the resource does not exist, a response code of 404 Not Found shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.17.3.2	DELETE

# This method shall support the URI query parameters specified in table 5.2.17.3.2-1.

# Table 5.2.17.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.17.3.2-2 and the response data structures and response codes specified in table 5.2.17.3.2-3.

# Table 5.2.17.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.17.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - USER_NOT_FOUND - DATA_NOT_FOUND |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.17.3.3	PATCH

This method shall support the URI query parameters specified in table 5.2.17.3.3-1.

# Table 5.2.17.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |

This method shall support the request data structures specified in table 5.2.17.3.3-2 and the response data structures and response codes specified in table 5.2.17.3.3-3.

# Table 5.2.17.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(PatchItem) | M | 1..N | Contains the delta data to the Individual SDM Subscription data. |

# Table 5.2.17.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon successful modification, there is no body in the response message. (NOTE 2) |
| PatchResult | M | 1 | 200 OK | Upon success, the execution report is returned. (NOTE 2) |
| ProblemDetails | O | 0..1 | 403 Forbidden | If one or more attributes are not allowed to be modified according to e.g. policy or local configuration, then the invalidParams attribute shall contain the JSON pointers of attributes which are not allowed to be modified, and the cause attribute shall be set to "MODIFICATION_NOT_ALLOWED", see 3GPP TS 29.500 [8] table 5.2.7.2-1. |
| NOTE 1: In addition common data structures as listed in table 5.5-1 are supported. NOTE 2: If all the modification instructions in the PATCH request have been implemented, the UDR shall respond with 204 No Content response; if some of the modification instructions in the PATCH request have been discarded, and the NF service consumer has included in the supported-feature query parameter the "PatchReport" feature number, the UDR shall respond with PatchResult. |  |  |  |  |

##### 5.2.17.3.4	GET

# This method shall support the URI query parameters specified in table 5.2.17.3.4-1.

# Table 5.2.17.3.4-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.17.3.4-2 and the response data structures and response codes specified in table 5.2.17.3.4-3.

# Table 5.2.17.3.4-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.17.3.4-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SdmSubscription | M | 1 | 200 OK | Upon success, a response body containing the individual SDM subscriptions shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

### 5.2.18	Resource: EeSubscriptions

#### 5.2.18.1	Description

This resource represents the collection of EE Subscriptions for a UE.

# This resource is modelled with the Collection resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

#### 5.2.18.2	Resource Definition

# Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/context-data/ee-subscriptions

# This resource shall support the resource URI variables defined in table 5.2.18.2-1.

# Table 5.2.18.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| ueId | Represents the Subscription Identifier SUPI or GPSI (see 3GPP TS 23.501 [4] clause 5.9.2) pattern: See pattern of type VarUeId 3GPP TS 29.571 [3] |

#### 5.2.18.3	Resource Standard Methods

##### 5.2.18.3.1	GET

# This method shall support the URI query parameters specified in table 5.2.18.3.1-1.

# Table 5.2.18.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [8] clause 6.6 |

This method shall support the request data structures specified in table 5.2.18.3.1-2 and the response data structures and response codes specified in table 5.2.18.3.1-3.

# Table 5.2.18.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.18.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(EeSubscription) | M | 1..N | 200 OK | Upon success, a response body containing the individual EE subscriptions shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.18.3.2	POST

This method shall support the URI query parameters specified in table 5.2.18.3.2-1.

# Table 5.2.18.3.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.18.3.1-2 and the response data structures and response codes specified in table 5.2.18.3.1-3.

# Table 5.2.18.3.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EeSubscription | M | 1 | Identifies the subscriber data subscription to be stored in the UDR. |

# Table 5.2.18.3.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EeSubscription | M | 1 | 201 Created | Upon success, a response body containing a representation of the UeId's Individual subscription resource shall be returned. The HTTP response shall include a "Location" HTTP header that contains the resource URI of the created resource. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

### 5.2.19	Resource: IndividualEeSubscription

#### 5.2.19.1	Description

This resource represents an individual EE Subscription for a UE.

# This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

#### 5.2.19.2	Resource Definition

# Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/context-data/ee-subscriptions/{subsId}

# This resource shall support the resource URI variables defined in table 5.2.19.2-1.

# Table 5.2.19.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| ueId | Represents the Subscription Identifier SUPI or GPSI (see 3GPP TS 23.501 [4] clause 5.9.2) pattern: See pattern of type VarUeId 3GPP TS 29.571 [3] |
| subsId | Identifier of the subscription (allocated by the UDR) |

#### 5.2.19.3	Resource Standard Methods

##### 5.2.19.3.1	PUT

# This method shall support the URI query parameters specified in table 5.2.19.3.1-1.

# Table 5.2.19.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.19.3.1-2 and the response data structures and response codes specified in table 5.2.19.3.1-3.

# Table 5.2.19.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EeSubscription | M | 1 | The individual EE subscription that is to be stored |

# Table 5.2.19.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| ProblemDetails | O | 0..1 | 404 Not Found | If the resource does not exist, a response code of 404 Not Found shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.19.3.2	DELETE

# This method shall support the URI query parameters specified in table 5.2.19.3.2-1.

# Table 5.2.19.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.19.3.2-2 and the response data structures and response codes specified in table 5.2.19.3.2-3.

# Table 5.2.19.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.19.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.19.3.3	PATCH

This method shall support the URI query parameters specified in table 5.2.19.3.3-1.

# Table 5.2.19.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |

This method shall support the request data structures specified in table 5.2.19.3.3-2 and the response data structures and response codes specified in table 5.2.19.3.3-3.

# Table 5.2.19.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(PatchItem) | M | 1..N | Contains the delta data to the Individual EE Subscription data |

# Table 5.2.19.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon successful modification, there is no body in the response message. (NOTE 2) |
| PatchResult | M | 1 | 200 OK | Upon success, the execution report is returned. (NOTE 2) |
| ProblemDetails | O | 0..1 | 403 Forbidden | If one or more attributes are not allowed to be modified according to e.g. policy or local configuration, then the invalidParams attribute shall contain the JSON pointers of attributes which are not allowed to be modified, and the cause attribute shall be set to "MODIFICATION_NOT_ALLOWED", see 3GPP TS 29.500 [8] table 5.2.7.2-1. |
| NOTE 1: In addition common data structures as listed in table 5.5-1 are supported. NOTE 2: If all the modification instructions in the PATCH request have been implemented, the UDR shall respond with 204 No Content response; if some of the modification instructions in the PATCH request have been discarded, and the NF service consumer has included in the supported-feature query parameter the "PatchReport" feature number, the UDR shall respond with PatchResult. |  |  |  |  |

##### 5.2.19.3.4	GET

# This method shall support the URI query parameters specified in table 5.2.19.3.4-1.

# Table 5.2.19.3.4-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.19.3.4-2 and the response data structures and response codes specified in table 5.2.19.3.4-3.

# Table 5.2.19.3.4-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.19.3.4-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EeSubscription | M | 1 | 200 OK | Upon success, a response body containing the individual EE subscriptions shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

### 5.2.19A	Resource: IndividualEeGroupSubscription

#### 5.2.19A.1	Description

This resource represents an individual EE Subscription for a group of UEs or any UE (i.e. all UEs).

# This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

#### 5.2.19A.2	Resource Definition

# Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/group-data/{ueGroupId}/ee-subscriptions/{subsId}

# This resource shall support the resource URI variables defined in table 5.2.19A.2-1.

# Table 5.2.19A.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| ueGroupId | Represents the identifier of the Group of UEs. For this resource, either External Group Identifier or any UE shall be present. Pattern: '^(extgroupid-[^@]+@[^@]+¦anyUE)$' |
| subsId | Identifier of the subscription (allocated by the UDM) |

#### 5.2.19A.3	Resource Standard Methods

##### 5.2.19A.3.1	PUT

# This method shall support the URI query parameters specified in table 5.2.19A.3.1-1.

# Table 5.2.19A.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.19A.3.1-2 and the response data structures and response codes specified in table 5.2.19A.3.1-3.

# Table 5.2.19A.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EeSubscription | M | 1 | The individual EE subscription for the group of UEs that is to be stored |

# Table 5.2.19A.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| ProblemDetails | O | 0..1 | 404 Not Found | If the resource does not exist, a response code of 404 Not Found shall be returned. |
| NOTE: In addition, common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.19A.3.2	DELETE

# This method shall support the URI query parameters specified in table 5.2.19A.3.2-1.

# Table 5.2.19A.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.19A.3.2-2 and the response data structures and response codes specified in table 5.2.19A.3.2-3.

# Table 5.2.19.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.19A.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| NOTE: In addition, common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.19A.3.3	PATCH

This method shall support the URI query parameters specified in table 5.2.19A.3.3-1.

# Table 5.2.19A.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |

This method shall support the request data structures specified in table 5.2.19A.3.3-2 and the response data structures and response codes specified in table 5.2.19A.3.3-3.

# Table 5.2.19A.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(PatchItem) | M | 1..N | Contains the delta data to the Individual EE Group Subscription data |

# Table 5.2.19A.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon successful modification, there is no body in the response message. (NOTE 2) |
| PatchResult | M | 1 | 200 OK | Upon success, the execution report is returned. (NOTE 2) |
| ProblemDetails | O | 0..1 | 403 Forbidden | If one or more attributes are not allowed to be modified according to e.g. policy or local configuration, then the invalidParams attribute shall contain the JSON pointers of attributes which are not allowed to be modified, and the cause attribute shall be set to "MODIFICATION_NOT_ALLOWED", see 3GPP TS 29.500 [8] table 5.2.7.2-1. |
| NOTE 1: In addition common data structures as listed in table 5.5-1 are supported. NOTE 2: If all the modification instructions in the PATCH request have been implemented, the UDR shall respond with 204 No Content response; if some of the modification instructions in the PATCH request have been discarded, and the NF service consumer has included in the supported-feature query parameter the "PatchReport" feature number, the UDR shall respond with PatchResult. |  |  |  |  |

##### 5.2.19A.3.4	GET

# This method shall support the URI query parameters specified in table 5.2.19A.3.4-1.

# Table 5.2.19A.3.4-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.19A.3.4-2 and the response data structures and response codes specified in table 5.2.19A.3.4-3.

# Table 5.2.19A.3.4-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.19A.3.4-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EeSubscription | M | 1 | 204 OK | Upon success, a response body containing the individual EE subscriptions for the groups of UEs shall be returned.. |
| NOTE: In addition, common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

### 5.2.20	Resource: SubscriptionDataSubscriptions

#### 5.2.20.1	Description

This resource is used to represent explicit subscriber data subscriptions to notifications.

#### 5.2.20.2	Resource Definition

Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/subs-to-notify

This resource shall support the resource URI variables defined in table 5.2.20.2-1.

# Table 5.2.20.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |

#### 5.2.20.3	Resource Standard Methods

##### 5.2.20.3.1	POST

This method shall support the URI query parameters specified in table 5.2.20.3.1-1.

# Table 5.2.20.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.20.3.1-2 and the response data structures and response codes specified in table 5.2.20.3.1-3.

# Table 5.2.20.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SubscriptionDataSubscriptions | M | 1 | Identifies the subscriber data subscription to be stored in the UDR. |

# Table 5.2.20.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SubscriptionDataSubscriptions | M | 1 | 201 Created | Upon success, a response body containing a representation of the UeId's Individual subscription resource shall be returned. The HTTP response shall include a "Location" HTTP header that contains the resource URI of the created resource. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.20.3.2	GET

# This method shall support the URI query parameters specified in table 5.2.20.3.2-1.

# Table 5.2.20.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ue-id | VarUeId | M | 1 | Represents the Subscription Identifier SUPI or GPSI (see 3GPP TS 23.501 [4] clause 5.9.2) pattern: See pattern of type VarUeId 3GPP TS 29.571 [3] |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [8] clause 6.6 |

This method shall support the request data structures specified in table 5.2.20.3.2-2 and the response data structures and response codes specified in table 5.2.20.3.2-3.

# Table 5.2.20.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.20.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(SubscriptionDataSubscriptions) | M | 0..N | 200 OK | Upon success, a response body containing the individual subscriptions shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.20.3.3	DELETE

This method shall support the URI query parameters specified in table 5.2.20.3.3-1.

# Table 5.2.20.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ue-id | VarUeId | M | 1 | Represents the Subscription Identifier SUPI or GPSI (see 3GPP TS 23.501 [4] clause 5.9.2) of the UE associated to this subscription. Pattern: See pattern of type VarUeId 3GPP TS 29.571 [3] |
| nf-instance-id | NfInstanceId | C | 0..1 | If included, this IE shall contain the Instance ID of the original NF Service Consumer of the UDM that created the subscription, and the UDR shall delete all subscriptions associated to the ue-id that were created by this specific nf-instance-id. If not included, the UDR shall delete all subscriptions matching the rest of query parameters. (See NOTE) |
| delete-all-nfs | boolean | C | 0..1 | If included, this IE indicates that the UDR shall delete all subscriptions associated to the ue-id. (See NOTE) |
| implicit-unsubscribe-indication | boolean | O | 0..1 | If included, this IE shall contain the content of the "implicitUnsubscribe" attribute sent during the creation of the subscription in UDM. If it is set to true, it means that the UDR shall delete all those subscriptions from NF Service Consumers of UDM, whose implicitUnsubscribe value is set to true. It not included, or set to false, the UDR shall delete all subscriptions matching the rest of query parameters. |
| NOTE: Either "nf-instance-id" or "delete-all-nfs" shal be included in the request. |  |  |  |  |

This method shall support the request data structures specified in table 5.2.20.3.3-2 and the response data structures and response codes specified in table 5.2.20.3.3-3.

# Table 5.2.20.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  | The request body shall be empty. |

# Table 5.2.20.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| NOTE: In addition, common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

### 5.2.21	Resource: IndividualSubscriptionDataSubscription

#### 5.2.21.1	Description

This resource is used to represent an individual subscriber data subscriptions to notifications.

#### 5.2.21.2	Resource Definition

Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/subs-to-notify/{subsId}

This resource shall support the resource URI variables defined in table 5.2.21.2-1.

# Table 5.2.21.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| subsId | The subsId identifies an individual SubscriptionDataSubscriptions to notifications. The value is allocated by the UDR during creation of the SubscriptionDataSubscriptions resource. |

#### 5.2.21.3	Resource Standard Methods

##### 5.2.21.3.1	DELETE

This method shall support the URI query parameters specified in table 5.2.21.3.1-1.

# Table 5.2.21.3.1-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.21.3.1-2 and the response data structures and response codes specified in table 5.2.21.3.1-3.

# Table 5.2.21.3.1-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  | The request body shall be empty. |

# Table 5.2.21.3.1-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.21.3.2	PATCH

This method shall support the URI query parameters specified in table 5.2.21.3.2-1.

# Table 5.2.21.3.2-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |

This method shall support the request data structures specified in table 5.2.21.3.2-2 and the response data structures and response codes specified in table 5.2.21.3.2-3.

# Table 5.2.21.3.2-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(PatchItem) | M | 1..N | Contains the delta data to the Individual Subscription Data Subscription. |

# Table 5.2.21.3.2-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon successful modification, there is no body in the response message. (NOTE 2) |
| PatchResult | M | 1 | 200 OK | Upon success, the execution report is returned. (NOTE 2) |
| ProblemDetails | O | 0..1 | 403 Forbidden | If one or more attributes are not allowed to be modified according to e.g. policy or local configuration, then the invalidParams attribute shall contain the JSON pointers of attributes which are not allowed to be modified, and the cause attribute shall be set to "MODIFICATION_NOT_ALLOWED", see 3GPP TS 29.500 [8] table 5.2.7.2-1. |
| NOTE 1: In addition common data structures as listed in table 5.5-1 are supported. NOTE 2: If all the modification instructions in the PATCH request have been implemented, the UDR shall respond with 204 No Content response; if some of the modification instructions in the PATCH request have been discarded, and the NF service consumer has included in the supported-feature query parameter the "PatchReport" feature number, the UDR shall respond with PatchResult. |  |  |  |  |

##### 5.2.21.3.3	GET

This method shall support the URI query parameters specified in table 5.2.21.3.3-1.

# Table 5.2.21.3.3-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.21.3.3-2 and the response data structures and response codes specified in table 5.2.21.3.3-3.

# Table 5.2.21.3.3-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  | The request body shall be empty. |

# Table 5.2.21.3.3-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SubscriptionDataSubscriptions | M | 1 | 200 OK | Upon success, a response body containing the individual subscriptions shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

### 5.2.22	Resource: TraceData

#### 5.2.22.1	Description

This resource represents the trace configuration data for a SUPI for use in a serving PLMN. It is queried by the AMF via the UDM after registering.

# This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

#### 5.2.22.2	Resource Definition

# Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/{servingPlmnId}/provisioned-data/trace-data

# This resource shall support the resource URI variables defined in table 5.2.22.2-1.

# Table 5.2.22.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| ueId | Represents the Subscription Identifier SUPI or GPSI (see 3GPP TS 23.501 [4] clause 5.9.2) pattern: See pattern of type VarUeId 3GPP TS 29.571 [3] |
| servingPlmnId | Represents the Serving PLMN ID (<MCC><MNC>) pattern: "^[0-9]{5,6}$" |

#### 5.2.22.3	Resource Standard Methods

##### 5.2.3.3.1	GET

# This method shall support the URI query parameters specified in table 5.2.22.3.1-1.

# Table 5.2.22.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.22.3.1-2 and the response data structures and response codes specified in table 5.2.22.3.1-3.

# Table 5.2.22.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TraceData | M | 1 | 200 OK | Upon success, a response body containing the Trace Configuration Data shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

#### 5.2.23	Resource: IdentityData

##### 5.2.23.1	Description

# This resource represents the identity data. It is queried by the UDM by GPSI or SUPI.

##### 5.2.23.2	Resource Definition

# Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/identity-data

# This resource shall support the resource URI variables defined in table 5.2.23.2-1.

# Table 5.2.23.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.1.1 |
| ueId | Represents the Subscription Identifier SUPI or GPSI (see 3GPP TS 23.501 [4] clause 5.9.2) pattern: See pattern of type VarUeId 3GPP TS 29.571 [3] |

##### 5.2.23.3	Resource Standard Methods

###### 5.2.23.3.1	GET

# This method shall support the URI query parameters specified in table 5.2.23.3.1-1.

# Table 5.2.23.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| app-port-id | AppPortId | O | 0..1 | Application port identity, see 3GPP TS 23.501 [2] clause 4.4.7 |

This method shall support the request data structures specified in table 5.2.23.3.1-2 and the response data structures and response codes specified in table 5.2.23.3.1-3.

# Table 5.2.23.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.23.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| IdentityData | M | 1 | 200 OK | Upon success, a response body containing the identity data that corresponds to the provided ueId |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

### 5.2.24	Resource: AuthenticationStatus

#### 5.2.24.1	Description

# This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

#### 5.2.24.2	Resource Definition

Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/authentication-data/authentication-status

This resource shall support the resource URI variables defined in table 5.2.24.2-1.

# Table 5.2.24.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| ueId | Represents the Subscription Identifier SUPI (see 3GPP TS 23.501 [4] clause 5.9.2) pattern: See pattern of type Supi in 3GPP TS 29.571 [3] |

#### 5.2.24.3	Resource Standard Methods

##### 5.2.24.3.1	PUT

This method shall support the URI query parameters specified in table 5.2.24.3.1-1.

# Table 5.2.24.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.24.3.1-2 and the response data structures and response codes specified in table 5.2.24.3.1-3.

# Table 5.2.24.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AuthEvent | M | 1 | To store the authentication status of the UE. |

# Table 5.2.24.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.24.3.2	GET

# This method shall support the URI query parameters specified in table 5.2.24.3.2-1.

# Table 5.2.24.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| fields | array(string) | C | 1..N | When the NF consumer only retrieves a subset of the resource, the "fields" query parameter shall be included. The "fields" query parameter contains the pointers of the attribute(s) to be retrieved. |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [8] clause 6.6 |

This method shall support the request data structures specified in table 5.2.24.3.2-2 and the response data structures and response codes specified in table 5.2.24.3.2-3.

# Table 5.2.24.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.24.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AuthEvent | M | 1 | 200 OK | Upon success, a response body containing the AuthEvent shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.24.3.3	DELETE

This method shall support the URI query parameters specified in table 5.2.24.3.3-1.

# Table 5.2.24.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.24.3.3-2 and the response data structures and response codes specified in table 5.2.24.3.3-3.

# Table 5.2.24.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.24.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

### 5.2.24A	Resource: IndividualAuthenticationStatus

#### 5.2.24A.1	Description

This resource is used to represent individual Authentication Status to be stored in the UDR.

# This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

#### 5.2.24A.2	Resource Definition

Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/authentication-data/authentication-status/{servingNetworkName}

This resource shall support the resource URI variables defined in table 5.2.24A.2-1.

# Table 5.2.24A.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.4.1 |
| ueId | Represents the Subscription Identifier SUPI (see 3GPP TS 23.501 [4] clause 5.9.2) pattern: See pattern of type Supi in 3GPP TS 29.571 [3] |
| servingNetworkName | Represents the Serving Network Name. pattern: See pattern of type ServingNetworkName in 3GPP TS 29.503 [6] |

#### 5.2.24A.3	Resource Standard Methods

##### 5.2.24A.3.1	PUT

This method shall support the URI query parameters specified in table 5.2.24A.3.1-1.

# Table 5.2.24A.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.24A.3.1-2 and the response data structures and response codes specified in table 5.2.24A.3.1-3.

# Table 5.2.24A.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AuthEvent | M | 1 | To store the authentication status of the UE. |

# Table 5.2.24A.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.24A.3.2	GET

# This method shall support the URI query parameters specified in table 5.2.24A.3.2-1.

# Table 5.2.24A.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| fields | array(string) | C | 1..N | When the NF consumer only retrieves a subset of the resource, the "fields" query parameter shall be included. The "fields" query parameter contains the pointers of the attribute(s) to be retrieved. |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [8] clause 6.6 |

This method shall support the request data structures specified in table 5.2.24A.3.2-2 and the response data structures and response codes specified in table 5.2.24A.3.2-3.

# Table 5.2.24A.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.24A.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AuthEvent | M | 1 | 200 OK | Upon success, a response body containing the AuthEvent shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.24A.3.3	DELETE

This method shall support the URI query parameters specified in table 5.2.24A.3.3-1.

# Table 5.2.24A.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.24A.3.3-2 and the response data structures and response codes specified in table 5.2.24A.3.3-3.

# Table 5.2.24A.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.24A.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

### 5.2.25	Resource: AuthenticationSoR

#### 5.2.25.1	Description

# This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

#### 5.2.25.2	Resource Definition

Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/ue-update-confirmation-data/sor-data

This resource shall support the resource URI variables defined in table 5.2.25.2-1.

# Table 5.2.25.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| ueId | Represents the Subscription Identifier SUPI (see 3GPP TS 23.501 [4] clause 5.9.2) pattern: See pattern of type Supi in 3GPP TS 29.571 [3] |

#### 5.2.25.3	Resource Standard Methods

##### 5.2.25.3.1	PUT

This method shall support the URI query parameters specified in table 5.2.25.3.1-1.

# Table 5.2.25.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.25.3.1-2 and the response data structures and response codes specified in table 5.2.25.3.1-3.

# Table 5.2.25.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SorData | M | 1 | The SoR acknowledgement information to be stored temporarily. |

# Table 5.2.25.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.25.3.2	GET

# This method shall support the URI query parameters specified in table 5.2.25.3.2-1.

# Table 5.2.25.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [8] clause 6.6 |

This method shall support the request data structures specified in table 5.2.25.3.2-2 and the response data structures and response codes specified in table 5.2.25.3.2-3.

# Table 5.2.25.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.25.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SorData | M | 1 | 200 OK | Upon success, a response body containing the SorData shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

### 5.2.25A	Resource: AuthenticationUPU

#### 5.2.25A.1	Description

# This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

#### 5.2.25A.2	Resource Definition

Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/ue-update-confirmation-data/upu-data

This resource shall support the resource URI variables defined in table 5.2.25A.2-1.

# Table 5.2.25A.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| ueId | Represents the Subscription Identifier SUPI (see 3GPP TS 23.501 [4] clause 5.9.2) pattern: See pattern of type Supi in 3GPP TS 29.571 [3] |

#### 5.2.25A.3	Resource Standard Methods

##### 5.2.25A.3.1	PUT

This method shall support the URI query parameters specified in table 5.2.25A.3.1-1.

# Table 5.2.25A.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.25A.3.1-2 and the response data structures and response codes specified in table 5.2.25A.3.1-3.

# Table 5.2.25A.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| UpuData | M | 1 | The UPU acknowledgement information to be stored temporarily. |

# Table 5.2.25A.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.25A.3.2	GET

# This method shall support the URI query parameters specified in table 5.2.25A.3.2-1.

# Table 5.2.25A.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [8] clause 6.6 |

This method shall support the request data structures specified in table 5.2.25A.3.2-2 and the response data structures and response codes specified in table 5.2.25A.3.2-3.

# Table 5.2.25A.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.25A.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| UpuData | M | 1 | 200 OK | Upon success, a response body containing the UpuData shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

### 5.2.25B	Resource: SubscribedSNSSAIs

#### 5.2.25B.1	Description

# This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

#### 5.2.25B.2	Resource Definition

Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/ue-update-confirmation-data/subscribed-snssais

This resource shall support the resource URI variables defined in table 5.2.25B.2-1.

# Table 5.2.25B.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| ueId | Represents the Subscription Identifier SUPI (see 3GPP TS 23.501 [4] clause 5.9.2) pattern: See pattern of type Supi in 3GPP TS 29.571 [3] |

#### 5.2.25B.3	Resource Standard Methods

##### 5.2.25B.3.1	PUT

This method shall support the URI query parameters specified in table 5.2.25B.3.1-1.

# Table 5.2.25B.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.25B.3.1-2 and the response data structures and response codes specified in table 5.2.25B.3.1-3.

# Table 5.2.25B.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| NssaiAckData | M | 1 | Contains the provisioning time stamp of the acknowledged subscribed S-NSSAIs |

# Table 5.2.25B.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.25B.3.2	GET

# This method shall support the URI query parameters specified in table 5.2.25B.3.2-1.

# Table 5.2.25B.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [8] clause 6.6 |

This method shall support the request data structures specified in table 5.2.25B.3.2-2 and the response data structures and response codes specified in table 5.2.25B.3.2-3.

# Table 5.2.25B.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.25B.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| NssaiAckData | M | 1 | 200 OK | Upon success, a response body containing the NssaiAckData shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

### 5.2.25C	Resource: SubscribedCAG

#### 5.2.25C.1	Description

# This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

#### 5.2.25C.2	Resource Definition

Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/ue-update-confirmation-data/subscribed-cag

This resource shall support the resource URI variables defined in table 5.2.25C.2-1.

# Table 5.2.25C.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| ueId | Represents the Subscription Identifier SUPI (see 3GPP TS 23.501 [4] clause 5.9.2) pattern: See pattern of type Supi in 3GPP TS 29.571 [3] |

#### 5.2.25C.3	Resource Standard Methods

##### 5.2.25C.3.1	PUT

This method shall support the URI query parameters specified in table 5.2.25C.3.1-1.

# Table 5.2.25C.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.25C.3.1-2 and the response data structures and response codes specified in table 5.2.25C.3.1-3.

# Table 5.2.25C.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| CagAckData | M | 1 | Contains the provisioning time stamp of the acknowledged CAG |

# Table 5.2.25C.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.25C.3.2	GET

# This method shall support the URI query parameters specified in table 5.2.25C.3.2-1.

# Table 5.2.25C.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [8] clause 6.6 |

This method shall support the request data structures specified in table 5.2.25C.3.2-2 and the response data structures and response codes specified in table 5.2.25C.3.2-3.

# Table 5.2.25C.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.25C.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| CagAckData | M | 1 | 200 OK | Upon success, a response body containing the CagAckData shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

### 5.2.26	Resource: ProvisionedData

#### 5.2.26.1	Description

This resource represents the subscribed ProvisionedData for a UE ID for use in a serving PLMN.

# This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

#### 5.2.26.2	Resource Definition

Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/{servingPlmnId}/provisioned-data

This resource shall support the resource URI variables defined in table 5.2.26.2-1.

# Table 5.2.26.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| ueId | Represents the Subscription Identifier SUPI or GPSI (see 3GPP TS 23.501 [4] clause 5.9.2) pattern: See pattern of type VarUeId in 3GPP TS 29.571 [3] |
| servingPlmnId | Represents the Serving PLMN ID (<MCC><MNC>) pattern: "^[0-9]{5,6}$" |

#### 5.2.26.3	Resource Standard Methods

##### 5.2.26.3.1	GET

This method shall support the URI query parameters specified in table 5.2.26.3.1-1.

# Table 5.2.26.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| dataset-names | array(DataSetName) | O | 1..N | If included, this IE shall contain the names of data sets to the retrieved. If not included, all provisioned data sets shall be returned. |

This method shall support the request data structures specified in table 5.2.26.3.1-2 and the response data structures and response codes specified in table 5.2.26.3.1-3.

# Table 5.2.26.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.26.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ProvisionedDataSets | M | 1 | 200 OK | Upon success, a response body containing the requested data sets shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

### 5.2.27	Resource: OperatorDeterminedBarringData

#### 5.2.27.1	Description

This resource represents the subscribed OperatorDeterminedBarringData for a SUPI. It is queried by the UDM during registration.

# This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

#### 5.2.27.2	Resource Definition

Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/operator-determined-barring-data

This resource shall support the resource URI variables defined in table 5.2.27.2-1.

# Table 5.2.27.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| ueId | Represents the Subscription Identifier SUPI or GPSI (see 3GPP TS 23.501 [4] clause 5.9.2) pattern: See pattern of type VarUeId in 3GPP TS 29.571 [3] |

#### 5.2.27.3	Resource Standard Methods

##### 5.2.27.3.1	GET

This method shall support the URI query parameters specified in table 5.2.27.3.1-1.

# Table 5.2.27.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.27.3.1-2 and the response data structures and response codes specified in table 5.2.27.3.1-3.

# Table 5.2.27.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.27.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| OdbData | M | 1 | 200 OK | Upon success, a response body containing the Operator Determined Barring Data shall be returned. |
| NOTE: In addition, common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

#### 5.2.28	Resource: EeProfileData

##### 5.2.28.1	Description

# This resource represents a list of the Event Exposure Profile Data. It is queried by the UDM by either GPSI or SUPI.

##### 5.2.28.2	Resource Definition

# Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/ee-profile-data

# This resource shall support the resource URI variables defined in table 5.2.28.2-1.

# Table 5.2.28.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.1.1 |
| ueId | Represents the Subscription Identifier SUPI or GPSI (see 3GPP TS 23.501 [4] clause 5.9.2) pattern: See pattern of type VarUeId in 3GPP TS 29.571 [3] |

##### 5.2.28.3	Resource Standard Methods

###### 5.2.28.3.1	GET

# This method shall support the URI query parameters specified in table 5.2.28.3.1-1.

# Table 5.2.28.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| fields | array(string) | C | 1-..N | When the NF consumer only retrieves a subset of the resource, the " fields " query parameter shall be included. The " fields " query parameter contains the pointers of the attribute(s) to be retrieved. |
| supportedFeatures | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [8] clause 6.6 |

This method shall support the request data structures specified in table 5.2.28.3.1-2 and the response data structures and response codes specified in table 5.2.28.3.1-3.

# Table 5.2.28.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.28.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EeProfileData | M | 1 | 200 OK | Upon success, a response body containing the Event Exposure Profile that corresponds to the provided ueId |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

### 5.2.29	Resource: EeGroupSubscriptions

#### 5.2.29.1	Description

This resource represents the collection of EE Subscriptions for a group of UEs or any UE (i.e. all UEs).

# This resource is modelled with the Collection resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

#### 5.2.29.2	Resource Definition

# Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/group-data/{ueGroupId}/ee-subscriptions

# This resource shall support the resource URI variables defined in table 5.2.29.2-1.

# Table 5.2.29.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| ueGroupId | Represents the identifier of the Group of UEs. For this resource, either External Group Identifier or any UE shall be present. pattern: '^(extgroupid-[^@]+@[^@]+¦anyUE)$' |

#### 5.2.29.3	Resource Standard Methods

##### 5.2.29.3.1	GET

# This method shall support the URI query parameters specified in table 5.2.29.3.1-1.

# Table 5.2.29.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [8] clause 6.6 |

This method shall support the request data structures specified in table 5.2.29.3.1-2 and the response data structures and response codes specified in table 5.2.29.3.1-3.

# Table 5.2.29.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.29.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(EeSubscription) | M | 1..N | 200 OK | Upon success, a response body containing the individual EE subscriptions for the groups of UEs shall be returned. |
| NOTE: In addition, common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.29.3.2	POST

This method shall support the URI query parameters specified in table 5.2.29.3.2-1.

# Table 5.2.29.3.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.19.3.2-2 and the response data structures and response codes specified in table 5.2.29.3.2-3.

# Table 5.2.29.3.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EeSubscription | M | 1 | Identifies the data subscription to be stored in the UDR. |

# Table 5.2.29.3.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EeSubscription | M | 1 | 201 Created | Upon success, a response body containing a representation of the Individual subscription resource shall be returned. The HTTP response shall include a "Location" HTTP header that contains the resource URI of the created resource. |
| NOTE: In addition, common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

### 5.2.30	Resource: SharedData

#### 5.2.30.1	Description

This resource represents the collection of data that can be shared by multiple UEs.

#### 5.2.30.2	Resource Definition

Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/shared-data

This resource shall support the resource URI variables defined in table 5.2.30.2-1.

# Table 5.2.30.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |

#### 5.2.30.3	Resource Standard Methods

##### 5.2.30.3.1	GET

This method shall support the URI query parameters specified in table 5.2.30.3.1-1.

# Table 5.2.30.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| shared-data-ids | array(SharedDataId) | M | 1..N |  |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [8] clause 6.6 |

This method shall support the request data structures specified in table 5.2.30.3.1-2 and the response data structures and response codes specified in table 5.2.30.3.1-3.

# Table 5.2.30.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.30.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(SharedData) | M | 1..N | 200 OK | Upon success, a response body containing a list of SharedData shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

### 5.2.31	Resource: AmfSubscriptionInfo

#### 5.2.31.1	Description

This resource represents information relevant to an individual EE Subscription for a UE. Depending on the Event Types of the EeSubscription, the UDM may need to consume the Namf_EventExposure_Subscribe service operation to subscribe at the AMF (see 3GPP TS 29.518 [13]). After successful subscription at the AMF, the UDM needs to store within the UDR related information, e.g. the SubscriptionId allocated by the AMF.

# This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

#### 5.2.31.2	Resource Definition

# Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/context-data/ee-subscriptions/{subsId}/amf-subscriptions

# This resource shall support the resource URI variables defined in table 5.2.31.2-1.

# Table 5.2.31.2-1: Resour ce URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| ueId | Represents the Subscription Identifier SUPI or GPSI (see 3GPP TS 23.501 [4] clause 5.9.2) pattern: See pattern of type VarUeId in 3GPP TS 29.571 [3] |
| subsId | Identifier of the subscription (allocated by the UDM) |

#### 5.2.31.3	Resource Standard Methods

##### 5.2.31.3.1	PUT

# This method shall support the URI query parameters specified in table 5.2.31.3.1-1.

# Table 5.2.31.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.31.3.1-2 and the response data structures and response codes specified in table 5.2.31.3.1-3.

# Table 5.2.31.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(AmfSubscriptionInfo) | M | 1..N | AMF subscription info for one or two AMFs |

# Table 5.2.31.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.31.3.2	DELETE

# This method shall support the URI query parameters specified in table 5.2.31.3.2-1.

# Table 5.2.31.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.31.3.2-2 and the response data structures and response codes specified in table 5.2.31.3.2-3.

# Table 5.2.31.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.31.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.31.3.3	GET

# This method shall support the URI query parameters specified in table 5.2.31.3.3-1.

# Table 5.2.31.3.3-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.31.3.3-2 and the response data structures and response codes specified in table 5.2.31.3.3-3.

# Table 5.2.31.3.3-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.31.3.3-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(AmfSubscriptionInfo) | M | 1..N | 200 OK | Upon success, a response body containing AMF Subscription Infos shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.31.3.4	PATCH

This method is used to modify the AMF subscription Infos in the UDR.

This method shall support the URI query parameters specified in table 5.2.31.3.4-1.

# Table 5.2.31.3.4-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |

This method shall support the request data structures specified in table 5.2.31.3.2-2 and the response data structures and response codes specified in table 5.2.31.3.4-3.

# Table 5.2.31.3.4-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(PatchItem) | M | 1..N | Contains the delta data to the provisioned parameter data |

# Table 5.2.31.3.4-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon successful modification there is no body in the response message. (NOTE 2) |
| PatchResult | M | 1 | 200 OK | Upon success, the execution report is returned. (NOTE 2) |
| ProblemDetails | O | 0..1 | 403 Forbidden | If one or more attributes are not allowed to be modified according to e.g. policy or local configuration, then the invalidParams attribute shall contain the JSON pointers of attributes which are not allowed to be modified, and the cause attribute shall be set to "MODIFICATION_NOT_ALLOWED", see 3GPP TS 29.500 [8] table 5.2.7.2-1. |
| NOTE 1: In addition common data structures as listed in table 5.5-1 are supported. NOTE 2: If all the modification instructions in the PATCH request have been implemented, the UDR shall respond with 204 No Content response; if some of the modification instructions in the PATCH request have been discarded, and the NF service consumer has included in the supported-feature query parameter the "PatchReport" feature number, the UDR shall respond with PatchResult. |  |  |  |  |

### 5.2.31A	Resource: SmfSubscriptionInfo

#### 5.2.31A.1	Description

This resource represents information relevant to an individual EE Subscription for a UE. Depending on the Event Types of the EeSubscription, the UDM may need to consume the Nsmf_EventExposure_Subscribe service operation to subscribe at the SMF (see 3GPP TS 29.508 [Y]). After successful subscription at the SMF, the UDM needs to store within the UDR related information, e.g. the SubscriptionId allocated by the SMF.

# This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

#### 5.2.31A.2	Resource Definition

# Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/context-data/ee-subscriptions/{subsId}/smf-subscriptions

# This resource shall support the resource URI variables defined in table 5.2.31A.2-1.

# Table 5.2.31A.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| ueId | Represents the Subscription Identifier SUPI or GPSI (see 3GPP TS 23.501 [4] clause 5.9.2) pattern: See pattern of type VarUeId in 3GPP TS 29.571 [3] |
| subsId | Identifier of the subscription (allocated by the UDM) |

#### 5.2.31A.3	Resource Standard Methods

##### 5.2.31A.3.1	PUT

# This method shall support the URI query parameters specified in table 5.2.31A.3.1-1.

# Table 5.2.31A.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.31A.3.1-2 and the response data structures, and response codes specified in table 5.2.31A.3.1-3.

# Table 5.2.31A.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SmfSubscriptionInfo | M | 1 | SMF subscription info |

# Table 5.2.31A.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| NOTE: In addition, common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.31A.3.2	DELETE

# This method shall support the URI query parameters specified in table 5.2.31A.3.2-1.

# Table 5.2.31A.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.31A.3.2-2 and the response data structures, and response codes specified in table 5.2.31A.3.2-3.

# Table 5.2.31A.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.31A.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| NOTE: In addition, common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.31A.3.3	GET

# This method shall support the URI query parameters specified in table 5.2.31A.3.3-1.

# Table 5.2.31A.3.3-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.31A.3.3-2 and the response data structures and response codes specified in table 5.2.31A.3.3-3.

# Table 5.2.31A.3.3-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.31A.3.3-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SmfSubscriptionInfo | M | 1 | 200 OK | Upon success, a response body containing SMF Subscription Infos shall be returned. |
| NOTE: In addition, common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.31A.3.4	PATCH

This method is used to modify the SMF subscription Infos in the UDR.

This method shall support the URI query parameters specified in table 5.2.31A.3.4-1.

# Table 5.2.31A.3.4-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |

This method shall support the request data structures specified in table 5.2.31A.3.2-2 and the response data structures, and response codes specified in table 5.2.31A.3.4-3.

# Table 5.2.31A.3.4-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(PatchItem) | M | 1..N | Contains the delta data to the provisioned parameter data |

# Table 5.2.31A.3.4-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon successful modification there is no body in the response message. (NOTE 2) |
| PatchResult | M | 1 | 200 OK | Upon success, the execution report is returned. (NOTE 2) |
| ProblemDetails | O | 0..1 | 403 Forbidden | If one or more attributes are not allowed to be modified according to e.g. policy or local configuration, then the invalidParams attribute shall contain the JSON pointers of attributes which are not allowed to be modified, and the cause attribute shall be set to "MODIFICATION_NOT_ALLOWED", see 3GPP TS 29.500 [8] table 5.2.7.2-1. |
| NOTE 1: In addition common data structures as listed in table 5.5-1 are supported. NOTE 2: If all the modification instructions in the PATCH request have been implemented, the UDR shall respond with 204 No Content response; if some of the modification instructions in the PATCH request have been discarded, and the NF service consumer has included in the supported-feature query parameter the "PatchReport" feature number, the UDR shall respond with PatchResult. |  |  |  |  |

### 5.2.32	Resource: ContextData

#### 5.2.32.1	Description

This resource represents the ContextData for a UE ID.

# This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

#### 5.2.32.2	Resource Definition

Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/context-data

This resource shall support the resource URI variables defined in table 5.2.32.2-1.

# Table 5.2.32.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| ueId | Represents the Subscription Identifier SUPI or GPSI (see 3GPP TS 23.501 [4] clause 5.9.2) pattern: See pattern of type VarUeId in 3GPP TS 29.571 [3] |

#### 5.2.32.3	Resource Standard Methods

##### 5.2.32.3.1	GET

This method shall support the URI query parameters specified in table 5.2.32.3.1-1.

# Table 5.2.32.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| context-dataset-names | array(ContextDataSetName) | M | 2..N | If included, this IE shall contain the names of context data sets to be retrieved. |

This method shall support the request data structures specified in table 5.2.32.3.1-2 and the response data structures and response codes specified in table 5.2.32.3.1-3.

# Table 5.2.32.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.32.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ContextDataSets | M | 1 | 200 OK | Upon success, a response body containing the requested context data sets shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

### 5.2.33	Resource: GroupIdentifiers

#### 5.2.33.1	Description

This resource represents the collection of data that can be shared by multiple UEs.

#### 5.2.33.2	Resource Definition

Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/group-data/group-identifiers

This resource shall support the resource URI variables defined in table 5.2.33.2-1.

# Table 5.2.33.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |

#### 5.2.33.3	Resource Standard Methods

##### 5.2.33.3.1	GET

This method shall support the URI query parameters specified in table 5.2.33.3.1-1.

# Table 5.2.33.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [8] clause 6.6 |
| ext-groud-id | ExtGroupId | C | 0..1 | External Group ID |
| int-group-id | GroupId | C | 0..1 | Internal Group ID |
| ue-id-ind | boolean | C | 0..1 | Indication whether UE identifiers are required or not. When present, it shall be set as following: - true: UE identifiers are required - false (default): UE identifiers are not required |
| NOTE: Either ext-group-id or int-group-id shall be present in the request. |  |  |  |  |

Either the ext-group-id or the int-group-id shall be present in the request.

This method shall support the request data structures specified in table 5.2.33.3.1-2 and the response data structures and response codes specified in table 5.2.33.3.1-3.

# Table 5.2.33.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.33.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| GroupIdentifiers | M | 1 | 200 OK | Upon success, a response body containing the group identifier(s) shall be returned. |
| NOTE: In addition, common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

### 5.2.34	Resource: 5GVnGroups

#### 5.2.34.1	Description

This resource represents the store of 5G VN Group data.

#### 5.2.34.2	Resource Definition

Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/group-data/5g-vn-groups

This resource shall support the resource URI variables defined in table 5.2.34.2-1.

# Table 5.2.34.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |

#### 5.2.34.3	Resource Standard Methods

##### 5.2.34.3.1	GET

This method shall support the URI query parameters specified in table 5.2.34.3.1-1.

# Table 5.2.34.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| gpsis | array(Gpsi) | O | 0..N | Only those group's data are retrieved of which at least one of the GPSIs is a member |

This method shall support the request data structures specified in table 5.2.34.3.1-2 and the response data structures and response codes specified in table 5.2.34.3.1-3.

# Table 5.2.34.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.34.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| map(5GVnGroupConfiguration) | M | 1 | 200 OK | Upon success, a response body containing the map (list of key-value pairs where ExtGroupId (see 3GPP TS 29.503 [6] serves as key) of 5GVnGroupConfiguration shall be returned. |
| NOTE: In addition, common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

### 5.2.35	Resource: Individual5GVnGroup

#### 5.2.35.1	Description

This resource represents an individual 5G VN Group.

# This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

#### 5.2.35.2	Resource Definition

# Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/group-data/5g-vn-groups/{externalGroupId}

# This resource shall support the resource URI variables defined in table 5.2.35.2-1.

# Table 5.2.35.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| externalGroupId | Represents the External Identifier of the group |

#### 5.2.35.3	Resource Standard Methods

##### 5.2.35.3.1	PUT

# This method shall support the URI query parameters specified in table 5.2.35.3.1-1.

# Table 5.2.35.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.35.3.1-2 and the response data structures and response codes specified in table 5.2.35.3.1-3.

# Table 5.2.35.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| 5GVnGroupConfiguration | M | 1 | See 3GPP TS 29.503 [6] |

# Table 5.2.35.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| 5GVnGroupConfiguration | M | 1 | 201 Created | Upon success, a representation of the created resource containing an internal group identifier as allocated by the UDR shall be returned. |
| ProblemDetails | O | 0..1 | 403 Forbidden | If the resource already exists, a response code of 403 Forbidden shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.35.3.2	DELETE

# This method shall support the URI query parameters specified in table 5.2.35.3.2-1.

# Table 5.2.35.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.2.35.3.2-2 and the response data structures and response codes specified in table 5.2.19.3.2-3.

# Table 5.2.35.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.35.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

##### 5.2.35.3.3	PATCH

This method shall support the URI query parameters specified in table 5.2.35.3.3-1.

# Table 5.2.35.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |

This method shall support the request data structures specified in table 5.2.35.3.3-2 and the response data structures and response codes specified in table 5.2.35.3.3-3.

# Table 5.2.35.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(PatchItem) | M | 1..N | Contains the delta data to the Individual 5GNvGroup |

# Table 5.2.35.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon successful modification, there is no body in the response message. (NOTE 2) |
| PatchResult | M | 1 | 200 OK | Upon success, the execution report is returned. (NOTE 2) |
| ProblemDetails | O | 0..1 | 404 Not Found | If the resource corresponding to the ExternalGroupId can't be found, the "cause" attribute shall be set to: - GROUP_IDENTIFIER_NOT_FOUND |
|  | O | 0..1 | 403 Forbidden | If one or more attributes are not allowed to be modified according to e.g. policy or local configuration, then the invalidParams attribute shall contain the JSON pointers of attributes which are not allowed to be modified, and the cause attribute shall be set to "MODIFICATION_NOT_ALLOWED", see 3GPP TS 29.500 [8] table 5.2.7.2-1 |
| NOTE 1: In addition common data structures as listed in table 5.5-1 are supported. NOTE 2: If all the modification instructions in the PATCH request have been implemented, the UDR shall respond with 204 No Content response; if some of the modification instructions in the PATCH request have been discarded, and the NF service consumer has included in the supported-feature query parameter the "PatchReport" feature number, the UDR shall respond with PatchResult. |  |  |  |  |

##### 5.2.35.3.4	GET

This method shall support the URI query parameters specified in table 5.2.35.3.4-1.

# Table 5.2.35.3.4-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [8] clause 6.6 |

This method shall support the request data structures specified in table 5.2.35.3.4-2 and the response data structures and response codes specified in table 5.2.35.3.4-3.

# Table 5.2.35.3.4-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.35.3.4-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| 5GVnGroupConfiguration | M | 1 | 200 OK | Upon success, a response body containing the group configuration of the individual 5G VN Group shall be returned. |
| NOTE: In addition, common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

### 5.2.36	Resource: LcsPrivacySubscriptionData

#### 5.2.36.1	Description

This resource represents the subscribed LcsPrivacySubscriptionData for an UE. It is queried by the HGMLC or NEF via the UDM.

# This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

#### 5.2.36.2	Resource Definition

Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/lcs-privacy-data

This resource shall support the resource URI variables defined in table 5.2.36.2-1.

# Table 5.2.36.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| ueId | Represents the Subscription Identifier SUPI or GPSI (see 3GPP TS 23.501 [4] clause 5.9.2) pattern: See pattern of type VarUeId in 3GPP TS 29.571 [3]. |

#### 5.2.36.3	Resource Standard Methods

##### 5.2.36.3.1	GET

This method shall support the URI query parameters specified in table 5.2.36.3.1-1.

# Table 5.2.36.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| fields | array(string) | C | 1..N | When the NF consumer only retrieves a subset of the resource, the " fields " query parameter shall be included. The " fields " query parameter contains the pointers of the attribute(s) to be retrieved. |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [8] clause 6.6 |

This method shall support the request data structures specified in table 5.2.36.3.1-2 and the response data structures and response codes specified in table 5.2.36.3.1-3.

# Table 5.2.36.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.36.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LcsPrivacyData | M | 1 | 200 OK | Upon success, a response body containing the LCS Privacy Subscription Data shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

### 5.2.37	Resource: LcsMobileOriginatedSubscriptionData

#### 5.2.37.1	Description

This resource represents the subscribed LcsMobileOriginatedSubscriptionData for a SUPI. It is queried by the AMF via the UDM after registering.

# This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

#### 5.2.37.2	Resource Definition

Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/lcs-mo-data

This resource shall support the resource URI variables defined in table 5.2.37.2-1.

# Table 5.2.37.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| ueId | Represents the Subscription Identifier SUPI or GPSI (see 3GPP TS 23.501 [4] clause 5.9.2) See pattern of type VarUeId in 3GPP TS 29.571 [3] |

#### 5.2.37.3	Resource Standard Methods

##### 5.2.37.3.1	GET

This method shall support the URI query parameters specified in table 5.2.37.3.1-1.

# Table 5.2.37.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| fields | array(string) | C | 1..N | When the NF consumer only retrieves a subset of the resource, the " fields " query parameter shall be included. The " fields " query parameter contains the pointers of the attribute(s) to be retrieved. |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [8] clause 6.6 |

This method shall support the request data structures specified in table 5.2.37.3.1-2 and the response data structures and response codes specified in table 5.2.37.3.1-3.

# Table 5.2.37.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.37.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LcsMoData | M | 1 | 200 OK | Upon success, a response body containing the LCS Mobile Originated Subscription Data shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

### 5.2.38	Resource: NiddAuthorizationData

#### 5.2.38.1	Description

This resource represents the subscribed NiddAuthorizationData for a GPSI. It is queried by the UDM during NIDD Authorization Data Retrieval and NIDD Authorization Data Update Notification.

# This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

#### 5.2.38.2	Resource Definition

Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/nidd-authorization-data

This resource shall support the resource URI variables defined in table 5.2.38.2-1.

# Table 5.2.38.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| ueId | Represents the Generic Public Subscription Identifier (see 3GPP TS 23.501 [4] clause 5.9.8) pattern: '^(msisdn-[0-9]{5,15}¦.+¦extid-[^@]+@[^@]+¦extgroupid-[^@]+@[^@]+)$' |

#### 5.2.38.3	Resource Standard Methods

##### 5.2.38.3.1	GET

This method shall support the URI query parameters specified in table 5.2.38.3.1-1.

# Table 5.2.38.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| single-nssai | Snssai | M | 1 | When present without Slice Differentiator (sd), all slices identified by the given Slice/Service Type (sst) and any sd value (if any) shall be considered matching the query parameter. |
| dnn | Dnn | M | 1 |  |
| mtc-provider-information | MtcProviderInformation | M | 1 |  |

This method shall support the request data structures specified in table 5.2.38.3.1-2 and the response data structures and response codes specified in table 5.2.38.3.1-3.

# Table 5.2.38.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.38.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AuthorizationData | M | 1 | 200 OK | Upon success, a response body containing the NIDD Authorization Data shall be returned. |
| NOTE: In addition, common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

### 5.2.39	Resource: CoverageRestrictionData

#### 5.2.39.1	Description

This resource represents UE's subscribed enhanced Coverage Restriction Data for a UE identifier. It is queried by the UDM during Enhanced Coverage Restriction Data Retrieval.

# This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

#### 5.2.39.2	Resource Definition

Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/coverage-restriction-data

This resource shall support the resource URI variables defined in table 5.2.39.2-1.

# Table 5.2.39.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| ueId | Represents the Generic Public Subscription Identifier (see 3GPP TS 23.501 [4] clause 5.9.8) pattern: '^(msisdn-[0-9]{5,15}¦extid-[^@]+@[^@]+¦.+)$' |

#### 5.2.39.3	Resource Standard Methods

##### 5.2.39.3.1	GET

This method shall support the URI query parameters specified in table 5.2.39.3.1-1.

# Table 5.2.39.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [8] clause 6.6 |

This method shall support the request data structures specified in table 5.2.39.3.1-2 and the response data structures and response codes specified in table 5.2.39.3.1-3.

# Table 5.2.39.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.39.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EnhancedCoverageRestrictionData | M | 1 | 200 OK | Upon success, a response body containing UE's subscribed enhanced Coverage Restriction Data. |
| NOTE: In addition, common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

### 5.2.40	Resource: Location

#### 5.2.40.1	Description

This resource represents the UE's Location Information for UE's identity (e.g. SUPI or GPSI).

# This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

#### 5.2.40.2	Resource Definition

Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/context-data/location

This resource shall support the resource URI variables defined in table 5.2.40.2-1.

# Table 5.2.40.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| ueId | Represents the Subscription Identifier SUPI or GPSI (see 3GPP TS 23.501 [4] clause 5.9.2) pattern: See pattern of type VarUeId in 3GPP TS 29.571 [3] |

#### 5.2.40.3	Resource Standard Methods

##### 5.2.40.3.1	GET

# This method shall support the URI query parameters specified in table 5.2.40.3.1-1.

# Table 5.2.40.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [8] clause 6.6 |

This method shall support the request data structures specified in table 5.2.40.3.1-2 and the response data structures and response codes specified in table 5.2.40.3.1-3.

# Table 5.2.40.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.40.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LocationInfo | M | 1 | 200 OK | Upon success, a response body containing the LocationInfo shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

### 5.2.41	Resource: V2xSubscriptionData

#### 5.2.41.1	Description

This resource represents UE's subscribed V2X Data for a UE identifier. It is queried by the UDM during V2X Subscription Data Retrieval.

# This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

#### 5.2.41.2	Resource Definition

Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/v2x-data

This resource shall support the resource URI variables defined in table 5.2.41.2-1.

# Table 5.2.41.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| ueId | Represents the Subscription Permanent Identifier (see 3GPP TS 23.501 [4] clause 5.9.2) or the Generic Public Subscription Identifier (see 3GPP TS 23.501 [4] clause 5.9.8). pattern: See pattern of type VarUeId in 3GPP TS 29.571 [3] |

#### 5.2.41.3	Resource Standard Methods

##### 5.2.41.3.1	GET

This method shall support the URI query parameters specified in table 5.2.41.3.1-1.

# Table 5.2.41.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [8] clause 6.6 |

This method shall support the request data structures specified in table 5.2.41.3.1-2 and the response data structures and response codes specified in table 5.2.41.3.1-3.

# Table 5.2.41.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.41.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| V2xSubscriptionData | M | 1 | 200 OK | Upon success, a response body containing UE's subscribed V2X Data. |
| NOTE: In addition, common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

### 5.2.42	Resource: LcsBroadcastAssistanceSubscriptionData

#### 5.2.42.1	Description

This resource represents the subscribed LcsBroadcast Assistance Subscription Data for an UE for use in a serving PLMN. It is queried by the LMF via the UDM.

# This resource is modelled with the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [7]).

#### 5.2.42.2	Resource Definition

Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/{servingPlmnId}/provisioned-data/lcs-bca-data

This resource shall support the resource URI variables defined in table 5.2.42.2-1.

# Table 5.2.42.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |
| ueId | Represents the Subscription Identifier SUPI or GPSI (see 3GPP TS 23.501 [4] clause 5.9.2 and 5.9.8) See pattern of type VarUeId in 3GPP TS 29.571 [3] |
| servingPlmnId | Represents the Serving PLMN ID (<MCC><MNC>) pattern: "^[0-9]{5,6}$" |

#### 5.2.42.3	Resource Standard Methods

##### 5.2.42.3.1	GET

This method shall support the URI query parameters specified in table 5.2.42.3.1-1.

# Table 5.2.42.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [8] clause 6.6 |

This method shall support the request data structures specified in table 5.2.42.3.1-2 and the response data structures and response codes specified in table 5.2.42.3.1-3.

# Table 5.2.42.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.42.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LcsBroadcastAssistanceTypesData | M | 1 | 200 OK | Upon success, a response body containing the LCS Broadcast Assistance Data Subscription Data shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

### 5.2.43	Resource: 5GVnGroupsInternal

#### 5.2.43.1	Description

This resource represents a read-only view of the collection resource of 5G VN Group data, so it can be queried based on the Internal Group Id from NF Service Consumers having such Id as the ony way to identify a group (e.g. PCF).

#### 5.2.43.2	Resource Definition

Resource URI: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/group-data/5g-vn-groups/internal

This resource shall support the resource URI variables defined in table 5.2.43.2-1.

# Table 5.2.43.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See 3GPP TS 29.504 [2] clause 6.1.1 |

#### 5.2.43.3	Resource Standard Methods

##### 5.2.43.3.1	GET

This method shall support the URI query parameters specified in table 5.2.43.3.1-1.

# Table 5.2.43.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| internal-group-ids | array(GroupId) | M | 1..N | Return those 5G VN groups whose Internal Group ID corresponds with any of the Group IDs in this list. |

This method shall support the request data structures specified in table 5.2.43.3.1-2 and the response data structures and response codes specified in table 5.2.43.3.1-3.

# Table 5.2.43.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.43.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| map(5GVnGroupConfiguration) | M | 1 | 200 OK | Upon success, a response body containing the map (list of key-value pairs where ExtGroupId (see 3GPP TS 29.503 [6] serves as key) of 5GVnGroupConfiguration shall be returned. |

## 5.3	Notifications

### 5.3.1	General

This clause describes the resources to provide Notification to NF service consumers which have explicitly or implicitly subscribed to UDR to be notified when data is changed.

### 5.3.2	Data Change Notification

The POST method shall be used for Data Change Notifications.

-	For explicit subscriptions (i.e., subscriptions created under the "subs-to-notif" collection resource, the URI shall be as provided during the subscription procedure;

-	For implicit subscriptions, the UDR shall use either a locally configured callback URI of the UDM, or the UDR may retrieve from NRF a default notification URI stored by UDM in NRF during the NF registration procedure.

-	If the UDM uses implicit subscriptions to receive notifications from UDR, the UDM shall register in NRF a default notification URI globally at NF Profile level (see 3GPP TS 29.510 [19], clause 6.1.6.2.2), rather than registering individual default notification URIs per NF Service.

The "callback" clause of the OpenAPI specification found in Annex A.2 associated to the POST method of the "subs-to-notif" resource shall be used as the notification request for both explicit and implicit subscriptions.

NOTE:	For explicit subscriptions, a stateless UDM/NF provides a URI which identifies a set of UDMs/NFs; a stateful UDM/NF provides a specific URI identifying a node.

Resource URI: {callbackReference}

Support of URI query parameters is specified in table 5.3.2-1.

# Table 5.3.2-1: URI query parameters supported by the POST method

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Support of request data structures is specified in table 5.3.2-2 and of response data structures and response codes is specified in table 5.3.2-3.

# Table 5.3.2-2: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DataChangeNotify | M | 1 |  |

# Table5.3.2-3: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |

### 5.3.3	Data Removal Notification

The POST method shall be used for Data Removal Notifications.

Data Removal Notifications are used for subscription withdraw cases implicitly subscribed by the UDM.

-	The UDR shall use either a locally configured callback URI of the UDM, or the UDR may retrieve from NRF a default notification URI stored by UDM in NRF during the NF registration procedure.

-	The UDM should register in NRF a default notification URI globally at NF Profile level, rather than registering individual default notification URIs per NF Service.

The "callback" clause of the OpenAPI specification found in Annex A.2 associated to the POST method of the "subs-to-notif" resource shall be used as the notification request.

Resource URI: {callbackReference}

Support of URI query parameters is specified in table 5.3.3-1.

# Table 5.3.3-1: URI query parameters supported by the POST method

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Support of request data structures is specified in table 5.3.3-2 and of response data structures and response codes is specified in table 5.3.3-3.

# Table 5.3.3-2: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DataChangeNotify | M | 1 | notifyItems within the DataChangeNotify type shall indicate deletion of the resources Amf3GppAccessRegistration and/or AmfNon3GppAccessRegistration |

# Table5.3.3-3: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

## 5.4	Data Model

### 5.4.1	General

This clause specifies the application data model supported by the API.

Table 5.4.1-1 specifies the data types defined for the Nudr service based interface protocol.

# Table 5.4.1-1: Nudr Subscriber Data specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| AuthenticationSubscription | 5.4.2.2 | A UE's authentication data |
| OperatorSpecificDataContainer | 5.4.2.3 | Container for operator specific data |
| SmfRegList | 5.4.2.4 | The list of all the SMF registrations of a UE |
| SubscriptionDataSubscriptions | 5.4.2.5 | A subscription to notifications. |
| DataChangeNotify | 5.4.2.6 | Container for data which have changed and notification was requested when changed. |
| IdentityData | 5.4.2.7 | Identity data corresponds to the provided ueId |
| ProvisionedDataSets | 5.4.2.8 |  |
| SorData | 5.4.2.9 | Used to store the status of the latest SOR data update |
| UpuData | 5.4.2.9A | Used to store the status of the latest UPU data update |
| NssaiAckData | 5.4.2.9B | Used to store the status of the latest NSSAI data update |
| CagAckData | 5.4.2.9C | Used to store the status of the latest CAG data update |
| AmfSubscriptionInfo | 5.4.2.19 | Information the UDR stores and retrieves related to active subscriptions at the AMF(s) |
| EeProfileData | 5.4.2.20 | Event Exposure Profile Data |
| ContextDataSets | 5.4.2.22 |  |
| SequenceNumber | 5.4.2.23 |  |
| MessageWaitingData | 5.4.2.24 |  |
| SmscData | 5.4.2.25 |  |
| SmfSubscriptionInfo | 5.4.2.26 | Information the UDR stores and retrieves related to active subscriptions at the SMF(s) |
| SmfSubscriptionItem | 5.4.2.27 | Contains info about a single SMF event subscription |
| MtcProvider | 5.4.2.28 |  |
| AuthMethod | 5.4.3.3 |  |
| DataSetName | 5.4.3.4 |  |
| ContextDataSetName | 5.4.3.6 |  |
| SqnScheme | 5.4.3.7 |  |
| Sign | 5.4.3.8 |  |
| UeUpdateStatus | 5.4.3.9 |  |

Table 5.4.1-2 specifies data types re-used by the Nudr service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nudr service based interface.

# Table 5.4.1-2: Nudr re-used Data Types

| ` |  | Reference |  | Comments |  |
| --- | --- | --- | --- | --- | --- |
| AccessAndMobilitySubscriptionData |  | 3GPP TS 29.503 [6] |  | Access and Mobility Subscription Data |  |
| SmfSelectionSubscriptionData |  | 3GPP TS 29.503 [6] |  | SMF Selection Subscription Data |  |
| SessionManagementSubscriptionData |  | 3GPP TS 29.503 [6] |  | Session Management Subscription Data |  |
| Amf3GppAccessRegistration |  | 3GPP TS 29.503 [6] |  |  |  |
| AmfNon3GppAccessRegistration |  | 3GPP TS 29.503 [6] |  | The complete set of information relevant to the AMF where the UE has registered via non 3GPP access. |  |
| SmfRegistration |  | 3GPP TS 29.503 [6] |  |  |  |
| SmsfRegistration |  | 3GPP TS 29.503 [6] |  |  |  |
| SmsManagementSubscriptionData |  | 3GPP TS 29.503 [6] |  |  |  |
| SupportedFeatures |  | 3GPP TS 29.571 [3] |  | Used to negotiate the applicability of the optional features |  |
| ProblemDetails |  | 3GPP TS 29.571 [3] |  | Detailed information about the status code. |  |
| PpData |  | 3GPP TS 29.503 [6] |  |  |  |
| SmsSubscriptionData |  | 3GPP TS 29.503 [6] |  |  |  |
| PatchItem |  | 3GPP TS 29.571 [3] |  | Data structure used for JSON patch. |  |
| SdmSubscription |  | 3GPP TS 29.503 [6] |  |  |  |
| EeSubscription |  | 3GPP TS 29.503 [6] |  |  |  |
| Uri |  | 3GPP TS 29.571 [3] |  |  |  |
| TraceData |  | 3GPP TS 29.571 [3] |  |  |  |
| Dnn |  | 3GPP TS 29.571 [3] |  | Data Network Name |  |
| Snssai |  | 3GPP TS 29.571 [3] |  | Single NSSAI |  |
| VarUeId |  | 3GPP TS 29.571 [3] |  | String represents the SUPI or GPSI. |  |
| NfInstanceId |  | 3GPP TS 29.571 [3] |  |  |  |
| NotifyItem |  | 3GPP TS 29.571 [3] |  | Common data type used for data change notification. |  |
| OdbData |  | 3GPP TS 29.571 [3] |  | Operator Determined Barring Data |  |
| EventType |  | 3GPP TS 29.503 [6] |  |  |  |
| ExtGroupId |  | 3GPP TS 29.503 [6] |  | External Group Identifier |  |
| PduSessionId |  | 3GPP TS 29.571 [3] |  |  |  |
| DateTime |  | 3GPP TS 29.571 [3] |  |  |  |
| UpuMac |  | 3GPP TS 29.509 [15] |  |  |  |
| 5GVnGroupConfiguration |  | 3GPP TS 29.503 [6] |  |  |  |
| PatchResult |  | 3GPP TS 29.571 [7] |  |  |  |
| SupportedFeatures |  | 3GPP TS 29.571 [7] |  |  |  |
| AppPortId |  | 3GPP TS 29.503 [6] |  |  |  |
| LcsPrivacyData |  | 3GPP TS 29.503 [6] |  | LCS Privacy Subscription Data |  |
| LcsMoData |  | 3GPP TS 29.503 [6] |  | LCS Mobile Originated Subscription Data |  |
| MtcProviderInformation |  | 3GPP TS 29.571 [3] |  |  |  |
| AuthorizationData |  | 3GPP TS 29.503 [6] |  |  |  |
| EnhancedCoverageRestrictionData |  | 3GPP TS 29.503 [6] |  |  |  |
| LocationInfo |  | 3GPP TS 29.503 [6] |  |  |  |
| V2xSubscriptionData |  | 3GPP TS 29.503 [6] |  | V2X Subscription Data |  |
| E164Number |  | 3GPP TS 29.503 [6] |  |  |  |
| NetworkNodeDiameterAddress |  | 3GPP TS 29.503 [6] |  |  |  |
| IpSmGwRegistration |  | 3GPP TS 29.503 [6] |  |  |  |
| LcsBroadcastAssistanceTypesData |  | 3GPP TS 29.503 [6] |  | LCS Broadcast Assistance Data Types |  |
| ContextInfo |  | 3GPP TS 29.503 [6] |  |  |  |
| NfGroupId |  | 3GPP TS 29.571 [3] |  |  |  |

### 5.4.2	Structured data types

#### 5.4.2.1	Introduction

This clause defines the structures to be used in resource representations.

#### 5.4.2.2	Type: AuthenticationSubscription

# Table 5.4.2.2-1: Definition of type AuthenticationSubscription

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| authenticationMethod | AuthMethod | M | 1 | String containing the Authentication Method ("5G_AKA", "EAP_AKA_PRIME", "EAP_TLS"...) that shall be used when the UE's device is 5G capable. If AV generation for HSS is required in UDM, this attribute shall take any 5G AKA-based value (i.e. "5G_AKA" or "EAP_AKA_PRIME"). |
| encPermanentKey | string | C | 0..1 | The encrypted value (hexstring) of the permanent authentication key (K) (see 3GPP TS 33.501 [9]). It shall be present if the authentication method is "5G_AKA" or "EAP_AKA_PRIME" unless vector generation is to be done in the HSS. |
| protectionParameterId | string | C | 0..1 | Identifies a parameter set securely stored in the UDM (ARPF) that can be used to decrypt the encPermanentKey (and encOpcKey or encTopcKey if present). Values and their meaning are HPLMN-operator specific. It shall be present if the authentication method is "5G_AKA" or "EAP_AKA_PRIME" unless vector generation is to be done in the HSS. |
| sequenceNumber | SequenceNumber | C | 0..1 | String containing the SQN as defined in 3GPP TS 33.102 [10]. It shall be present if the authentication method is "5G_AKA" or "EAP_AKA_PRIME" unless vector generation is to be done in the HSS. |
| authenticationManagementField | string | C | 0..1 | Hexstring containing the Authentication management field as defined in 3GPP TS 33.501 [9]. It shall be present if the authentication method is "5G_AKA" or "EAP_AKA_PRIME" unless vector generation is to be done in the HSS. Pattern: '^[A-Fa-f0-9]{4}$' |
| algorithmId | string | C | 0..1 | Identifies a parameter set securely stored in the UDM (ARPF) that provides details on the algorithm and parameters used to generate authentication vectors. Values and their meaning are HPLMN-operator specific. It shall be present if the authentication method is "5G_AKA" or "EAP_AKA_PRIME" unless vector generation is to be done in the HSS. |
| encOpcKey | string | O | 0..1 | Hexstring of the encrypted OPC Key. Presence indicates that the provided value (decrypted) shall be used instead of the value derived from OP and K. |
| encTopcKey | string | O | 0..1 | Hexstring of the encrypted TOPC Key. Presence indicates that the provided value (decrypted) shall be used instead of the value derived from TOP and K. |
| hssGroupId | NfGroupId | O | 0..1 | Identity of the HSS group associated with the subscription that might be used by the UDM to discover the HSS; see 3GPP TS 29.510 [19]. This attribute may be present if the vectorGenerationInHss attribute is present and set to True. |
| vectorGenerationInHss | boolean | O | 0..1 | True indicates that the UDM needs to retrieve an Authentication Vector from the HSS; False and absence indicates that vector generation shall be performed in the UDM. |
| n5gcDeviceAuthMethod | AuthMethod | O | 0..1 | String containing the Authentication Method that shall be used when the UE's device is Non-5G-Capable behind Cable RGs in private networks or in isolated deployment scenarios with wireline access. See NOTE. |
| rgAuthenticationInd | boolean | O | 0..1 | true: indicates that authentication by the home network is not required if authentication has been completed by the W-5GAN. See 3GPP TS 33.501 [9] clause 7b. false (default): otherwise. |
| supi | Supi | C | 0..1 | If present, this IE shall contain the UE's SUPI which shall contain an IMSI. It shall be present if the subscription is allowed to be identified by a pseudonym of the SUPI e.g. by a GCI or GLI that is not the SUPI. |
| akmaAllowed | boolean | O | 0..1 | This IE indicates whether or not the subscriber is allowed to use AKMA: - true: subscriber is allowed to use AKMA - absent or false: subscriber is not allowed to use AKMA |
| NOTE: The attribute n5gcDeviceAuthMethod is used for EAP-TLS, which is described in the informative annex O of 3GPP TS 33.501 [9] and is not mandatory to support. |  |  |  |  |

#### 5.4.2.3	Type: OperatorSpecificDataContainer

# Table 5.4.2.3-1: Definition of type OperatorSpecificDataContainer

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| dataType | string | M | 1 | This attribute value must be a string equals to the name of the actual data type of the "Value" attribute. |
| value | any type | M | 1 |  |
| dataTypeDefinition | string | O | 0..1 | If present, this attribute is applicable when "dataType" takes value "object" and it shall contain a JSON Pointer expression referencing a JSON Schema data type definition (same as how the attribute "$ref" is used in JSON Schema specifications), which the "value" attribute must conform to. |
| supportedFeatures | SupportedFeatures | C | 0..1 | Indicates the list of negotiated supported features. This parameter shall be supplied by the UDR in the response to the GET request when the GET request includes the "supported-features" query parameter and the UDR supports feature negotiation for the OperatorSpecificData resource defined in this specification and in 3GPP TS 29.519 [17]. |

EXAMPLE:	A JSON object of type "OperatorSpecificDataContainer" can be as follows:

{

"dataType": "object",

"value": {

"operatorData1": "foo",

"operatorData2": 123,

"operatorData3": false

},

"dataTypeDefinition": "https://operator.com/definitions.json#/OpSpecificDataType"

}

And where the file "/definitions.json" contains a fragment as follows:

{

# "OpSpecificDataType": {

"type": "object",

"properties": {

"operatorData1": {

"type": "string"

},

"operatorData2": {

"type": "integer"

},

"operatorData3": {

"type": "boolean"

}

}

}

}

#### 5.4.2.4	Type: SmfRegList

# Table 5.4.2.4-1: Definition of type SmfRegList

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A | array(SmfRegistration) | M | 1..N | Each member of the array is an individual SMF registration of the UE. |

#### 5.4.2.5	Type: SubscriptionDataSubscriptions

# Table 5.4.2.5-1: Definition of type SubscriptionDataSubscriptions

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| callbackReference | Uri | M | 1 | Identifies the UDM/NF pool or an individual UDM/NF which should be used by the UDR to send notification to UDM/NF. |
| originalCallbackReference | Uri | O | 0..1 | URI provided by the NF service consumer to the UDM to receive notifications from the UDR. If "sdmSubscription" is present, this attribute may be absent (and, if it is also present, it shall take the same value as the attribute "callbackReference" of "sdmSubscription"). |
| ueId | VarUeId | O | 0..1 | User identity if subscription to notifications is related to a user. |
| monitoredResourceUris | array(Uri) | M | 1..N | A set of URIs that identify the resources for which a modification of the representation triggers a notification. The URI shall take the form of either an absolute URI or an absolute-path reference as defined in IETF RFC 3986 [16]. |
| expiry | DateTime | C | 0..1 | This IE shall be included in a subscription response, if, based on operator policy and taking into account the expiry time included in the request, the UDR needs to include an expiry time. This IE may be included in a subscription request. When present, this IE shall represent the time after which the subscription becomes invalid. The absence of this attribute in the subscription response means the subscription to be valid without an expiry time. |
| sdmSubscription | SdmSubscription | O | 0..1 | Data related to the subscription to changes on SDM data, created by the NF Service Consumer of the UDM (see 3GPP TS 29.503 [6], clause 6.1.6.2.3). |
| subscriptionId | string | C | 0..1 | This attribute shall be present if the SubscriptionDataSubscriptions is sent in a GET response message or in a Notification. It identifies the individual SubscriptionDataSubscriptions stored in the UDR and may be used by the UDM to e.g. delete or update a SubscriptionDataSubscriptions. Read Only: true |
| supportedFeatures | SupportedFeatures | O | 0..1 | Used to negotiate the applicability of optional features |

#### 5.4.2.6	Type: DataChangeNotify

# Table 5.4.2.6-1: Definition of type DataChangeNotify

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| originalCallbackReference | array(Uri) | C | 1..N | Original Callback Reference shall be present if a UDM has subscribed to notification due to an request from an NF. If present it is Identifying the NF to which the UDM has to provide the notification of data change. |
| ueId | VarUeId | C | 0..1 | User identity if subscription to notifications is related to a user that may be provided e.g. in case OriginalCallback URI is not included. |
| notifyItems | array(NotifyItem) | O | 1..N | This attribute contains the changes which have been applied on the resources. |
| sdmSubscription | SdmSubscription | O | 0..1 | Data related to the subscription to changes on SDM data created by the NF Service Consumer of the UDM (see 3GPP TS 29.503 [6], clause 6.1.6.2.3). This attribute should be omitted if the SubscriptionsDataSubscriptions array is included. |
| additionalSdmSubscriptions | array(SdmSubscription) | C | 1..N | Shall be present if more than one subscription to changes on SDM data created by different NF service consumers of the UDM exist. This attribute if present shall convey sdm subscriptions in addition to and different from the sdm subscription present in the sdmSubscription attribute. |
| subscriptionDataSubscriptions | array(SubscriptionDataSubscriptions) | O | 1..N | This attribute contain the Subscription Data Subscriptions (subs-to-notify) associated with the Notification. |

#### 5.4.2.7	Type: IdentityData

# Table 5.4.2.7-1: Definition of type IdentityData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supiList | array(Supi) | C | 1..N | Supi list corresponding to the provided gpsi ueId. |
| gpsiList | array(Gpsi) | C | 1..N | Gpsi list corresponding to the provided supi ueId. Zero or one MSISDN and/or zero or more external Identifiers may be included. |

### 5.4.2.8	Type: ProvisionedDataSets

# Table 5.4.2.8-1: ProvisionedDataSets

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| amData | AccessAndMobilitySubscriptionData | O | 0..1 | Access and Mobility Subscription Data |
| smfSelData | SmfSelectionSubscriptionData | O | 0..1 | SMF Selection Subscription Data |
| smsSubsData | SmsSubscriptionData | O | 0..1 | SMS Subscription Data |
| smData | array(SessionManagementSubscriptionData) | O | 1..N | Session Management Subscription Data |
| traceData | TraceData | O | 0..1 | Trace Data. The Null value indicates that trace is not active. |
| smsMngData | SmsManagementSubscriptionData | O | 0..1 | SMS Management Subscription Data |
| lcsPrivacyData | LcsPrivacyData | O | 0..1 | LCS Privacy Subscription Data |
| lcsMoData | LcsMoData | O | 0..1 | LCS Mobile Originated Subscription Data |
| lcsBcaData | LcsBroadcastAssistanceTypesData | O | 0..1 | LCS Broadcast Assistance Subscription Data |
| v2xData | V2xSubscriptionData | O | 0..1 | V2X Subscription Data |

#### 5.4.2.9	Type: SorData

# Table 5.4.2.9-1: Definition of type SorData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| provisioningTime | DateTime | M | 1 | Point in time of provisioning |
| ueUpdateStatus | UeUpdateStatus | M | 1 | status of the SOR data update procedure |
| sorXmacIue | SorMac | C | 0..1 | String containing SoR-XMAC-IUE as specified in 3GPP TS 33.501 [9]. Shall be present if UeUpdateStatus is "WAITING_FOR_ACK" or "ACK_RECEIVED". |
| sorMacIue | SorMac | C | 0..1 | String containing SoR-MAC-IUE as specified in 3GPP TS 33.501 [9]. Shall be preset if UeUpdateStatus is "ACK_RECEIVED". |

#### 5.4.2.9A	Type: UpuData

# Table 5.4.2.9A-1: Definition of type UpuData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| provisioningTime | DateTime | M | 1 | Point in time of provisioning |
| ueUpdateStatus | UeUpdateStatus | M | 1 | status of the UPU data update procedure |
| upuXmacIue | UpuMac | C | 0..1 | String containing UPU-XMAC-IUE as specified in 3GPP TS 33.501 [9]. Shall be present if UeUpdateStatus is "WAITING_FOR_ACK" or "ACK_RECEIVED". |
| upuMacIue | UpuMac | C | 0..1 | String containing UPU-MAC-IUE as specified in 3GPP TS 33.501 [9]. Shall be preset if UeUpdateStatus is "ACK_RECEIVED". |

#### 5.4.2.9B	Type: NssaiAckData

# Table 5.4.2.9B-1: Definition of type NssaiAckData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| provisioningTime | DateTime | M | 1 | Point in time of provisioning |
| ueUpdateStatus | UeUpdateStatus | M | 1 | status of the Nssai update procedure |

#### 5.4.2.9C	Type: CagAckData

# Table 5.4.2.9C-1: Definition of type CagAckData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| provisioningTime | DateTime | M | 1 | Point in time of provisioning |
| ueUpdateStatus | UeUpdateStatus | M | 1 | status of the Cag update procedure |

#### 5.4.2.10	Void

#### 5.4.2.11	Void

#### 5.4.2.12	Void

#### 5.4.2.13	Void

#### 5.4.2.14	Void

#### 5.4.2.15	Void

#### 5.4.2.16	Void

#### 5.4.2.17	Void

#### 5.4.2.18	Void

#### 5.4.2.19	Type: AmfSubscriptionInfo

# Table 5.4.2.19-1: Definition of type AmfSubscriptionInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| amfInstanceId | NfInstanceId | M | 1 | InstanceId of the AMF to which the subscription was sent |
| subscriptionId | Uri | M | 1 | The Subscription Id allocated by the AMF as received by the UDM in the HTTP "Location" header of the Namf_EventExposure_Subscribe response |
| subsChangeNotifyCorrelationId | string | O | 1 | The Correlation Id allocated by the UDM and sent to the AMF for correlation of subscriptionId-change-notifications |
| contextInfo | ContextInfo | O | 0..1 | This IE if present may contain e.g. the headers received by the UDM along with the SdmSubscription. |

#### 5.4.2.20	Type: EeProfileData

# Table 5.4.2.20-1: Definition of type EeProfileData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| restrictedEventTypes | array(EventType) | O | 1..N | List of the event types that are restricted (if any) for the UE. The absence of this IE indicates that all event types are authorized for the UE. |
| supportedFeatures | SupportedFeatures | O | 0..1 |  |
| allowedMtcProvider | map(array(MtcProvider)) | O | 1..N | A map (list of key-value pairs where EventType (see 3GPP TS 29.503 [6] serves as key) of MTC Provider. In addition to defined EventTypes, the key value "ALL" may be used to identify a map entry which contains a list of MtcProviders that are allowed monitoring all Event Types. The absence of this IE indicates that monitoring by any MTC provider is allowed for any non restricted event type. |
| iwkEpcRestricted | boolean | O | 0..1 | If set to True, indicates that interworking with EPC is restricted. Set to False or not present means that the UE is not restriceted in the EPC. |
| imsi | string | O | 0..1 | This IE contains the IMSI of the UE and should be present if the iwkEpcRestricted attribute is not set to True. pattern: "^[0-9]{5,15}$" |
| hssGroupId | NfGroupId | O | 0..1 | Identity of the HSS group associated with the subscription that might be used by the UDM to discover the HSS; see 3GPP TS 29.510 [19]. This attribute may be present if the iwkEpcRestricted attribute is not present or present and set to False. |

#### 5.4.2.21	Void

#### 5.4.2.22	Type: ContextDataSets

# Table 5.4.2.22-1: ContextDataSets

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| amf3Gpp | Amf3GppAccessRegistration | O | 0..1 | AMF 3GPP Access registration |
| amfNon3Gpp | AmfNon3GppAccessRegistration | O | 0..1 | AMF Non 3GPP Access registration |
| sdmSubscriptions | array(SdmSubscription) | O | 1..N | SDM-Subscriptions |
| eeSubscriptions | array(EeSubscription) | O | 1..N | Event Exposure Subscriptions |
| smsf3GppAccess | SmsfRegistration | O | 0..1 | SMSF 3GPP Access registration |
| smsfNon3GppAccess | SmsfRegistration | O | 0..1 | SMSF Non 3GPP Access registration |
| subscriptionDataSubscriptions | array(SubscriptionDataSubscriptions) | O | 1..N | This attribute contain the Subscription Data Subscriptions (subs-to-notify) associated with the Notification. |
| smfRegistrations | SmfRegList | O | 0..1 | SMF Registrations |
| ipSmGw | IpSmGwRegistration | O | 0..1 | IP-SM-GW Registration |

#### 5.4.2.23	Type: SequenceNumber

# Table 5.4.2.23-1: Definition of type SequenceNumber

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| sqnScheme | SqnScheme | M | 1 | The scheme used to generate the sequence numbers, as described in 3GPP TS 33.102 [10], clause C.1.1. |
| sqn | string | C | 0..1 | A 48-bit hex string containing the SEQ part of SQN (most significant bits), as specified in 3GPP TS 33.102 [10], and where the IND part (least significant bits) is filled with 0's. When the sqnScheme is "TIME_BASED", the SEQ part (most significant bits) of this attribute contains the DIF value. This IE may be absent, if it does not exist in UDR (e.g. right after the subscriber is provisioned); otherwise, it shall be present. Pattern: '^[A-Fa-f0-9]{12}$' |
| lastIndexes | map(integer) | C | 1..N | A map of integer values, where the key of the map is the type of node that is requesting the generation of an authentication vector in UDM, and the integer is the last used value of IND for the corresponding type of node. The types of nodes currently defined are: - "ausf" - "vlr" - "sgsn" - "s-cscf" - "bsf", - "gan-aaa-server" - "wlan-aaa-server" - "mme" This IE may be absent, if it does not exist in UDR (e.g. right after the subscriber is provisioned); otherwise, it shall be present. |
| indLength | integer | O | 1 | Number of bits of the IND part of SQN. The length of the SEQ part is, therefore, (48 - indLength) bits. If not present, the default value is 5. |
| difSign | Sign | O | 1 | Sign of the DIF value. It is applicable when the sqnScheme is "TIME_BASED". If not present, the default value is "NEGATIVE". |

#### 5.4.2.24	Type: MessageWaitingData

# Table 5.4.2.24-1: Definition of type MessageWaitingData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| mwdList | array(SmscData) | O | 1..N | List of addresses of SM-Service Center entities with SMS wating to be delivered to the UE. |

#### 5.4.2.25	Type: SmscData

# Table 5.4.2.25-1: Definition of type SmscData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| smscMapAddress | E164Number | C | 0..1 | International E.164 number of the SM-SC; it shall be present if the SM-SC supports the MAP protocol. |
| smscDiameterAddress | NetworkNodeDiameterAddress | C | 0..1 | Diameter Identity of the SM-SC; it shall be present if the SM-SC supports the Diameter protocol. |
| NOTE: At least one of the properties, smscMapAddress or smscDiameterAddress, shall be present. |  |  |  |  |

#### 5.4.2.26	Type: SmfSubscriptionInfo

# Table 5.4.2.26-1: Definition of type SmfSubscriptionInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| smfSubscriptionList | array(SmfSubscriptionItem) | M | 1..N | List of SmfSubscriptionItem objects |

#### 5.4.2.27	Type: SmfSubscriptionItem

# Table 5.4.2.27-1: Definition of type SmfSubscriptionItem

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| smfInstanceId | NfInstanceId | M | 1 | InstanceId of the SMF to which the subscription was sent |
| subscriptionId | Uri | M | 1 | The Subscription Id allocated by the SMF as received by the UDM in the HTTP "Location" header of the Nsmf_EventExposure_Subscribe response |

#### 5.4.2.28	Type: MtcProvider

# Table 5.4.2.28-1: Definition of type MtcProvider

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| mtcProviderInformation | MtcProviderInformation | O | 0..1 | Indicates MTC provider information for Monitoring Configuration authorization. |
| afId | string | O | 0..1 | ID of the AF. The string identifying the originating AF, formatted as described in the definition of type MonitoringConfiguration in 3GPP TS 29.503 [6]. |

### 5.4.3	Simple data types and enumerations

#### 5.4.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

#### 5.4.3.2	Simple data types

The simple data types defined in table 5.4.3.2-1 shall be supported.

# Table 5.4.3.2-1: Simple data types

| Type Name | Type Definition | Description |
| --- | --- | --- |

#### 5.4.3.3	Enumeration: AuthMethod

# Table 5.4.3.3-1: Enumeration AuthMethod

| Enumeration value | Description |
| --- | --- |
| "5G_AKA" | 5G AKA |
| "EAP_AKA_PRIME" | EAP AKA' |
| "EAP-TLS" | EAP TLS |

#### 5.4.3.4	Enumeration: DataSetName

# Table 5.4.3.4-1: Enumeration DataSetName

| Enumeration value | Description |
| --- | --- |
| "AM" | Access and Mobility Subscription Data |
| "SMF_SEL" | SMF Selection Subscription Data |
| "SMS_SUB" | SMS Subscription Data |
| "SM" | Session Management Subscription Data |
| "TRACE" | Trace Data |
| "SMS_MNG" | SMS Management Subscription Data |
| "LCS_PRIVACY" | LCS Privacy Subscription Data |
| "LCS_MO" | LCS Mobile Originated Subscription Data |
| "LCS_BCA" | LCS Broadcast Assistance Subscription Data |
| "V2X" | V2X Subscription Data |

#### 5.4.3.5	Void

#### 5.4.3.6	Enumeration: ContextDataSetName

# Table 5.4.3.6-1: Enumeration ContextDataSetName

| Enumeration value | Description |
| --- | --- |
| "AMF_3GPP" | AMF 3GPP Access Registration |
| "AMF_NON_3GPP" | SMF Non 3GPP Access Registration |
| "SDM SUBSCRIPTIONS" | SDM Subscriptions |
| "EE_SUBSCRIPTIONS" | EE Subscriptions |
| "SMSF_3GPP" | SMSF 3GPP Access Registration |
| "SMSF_NON3GPP" | SMSF Non 3GPP Access Registration |
| "SUBS_TO_NOTIFY" | UDR Subscriptions |
| "SMF_REG" | SMF Registrations |
| "IP_SM_GW" | IP-SM-GW Registration |

#### 5.4.3.7	Enumeration: SqnScheme

# Table 5.4.3.7-1: Enumeration SqnScheme

| Enumeration value | Description |
| --- | --- |
| "GENERAL" | Scheme for generation of Sequence Numbers (partially time-based), as described in 3GPP TS 33.102 [10], clause C.1.1.1 |
| "NON_TIME_BASED" | Scheme for generation of Sequence Numbers (non-time-based), as described in 3GPP TS 33.102 [10], clause C.1.1.2 |
| "TIME_BASED" | Scheme for generation of Sequence Numbers (entirely time-based), as described in 3GPP TS 33.102 [10], clause C.1.1.3 |
| NOTE: These schemes for Sequence Number generation are documented in an informative Annex in 3GPP TS 33.102 [10]; having these different schemes listed in this Enumeration type does not imply that UDM implementations are required to support all of them; the Sequence Number generation schemes that need to be supported by UDM is deployment-specific. |  |

#### 5.4.3.8	Enumeration: Sign

# Table 5.4.3.8-1: Enumeration Sign

| Enumeration value | Description |
| --- | --- |
| "POSITIVE" |  |
| "NEGATIVE" |  |

#### 5.4.3.9	Enumeration: UeUpdateStatus

# Table 5.4.3.9-1: Enumeration UeUpdateStatus

| Enumeration value | Description |
| --- | --- |
| "NOT_SENT" | SOR data update / UPU data update / NSSAI update / CAG update has not been sent to the AMF e.g. because no AMF was registered at provisioning time |
| "SENT_NO_ACK_REQUIRED" | SOR data update / UPU data update has been sent to the AMF; Acknowledgement was not requested This state is not applicable for NSSAI update / CAG update |
| "WAITING_FOR_ACK" | SOR data update has been sent to the AMF; Acknowledgement has been requested. In this state sorXmacIue shall be present; or UPU data update has been sent to the AMF; Acknowledgement has been requested. In this state upuXmacIue shall be present; or NSSAI has been sent to the AMF or CAG has been sent to the AMF. |
| "ACK_RECEIVED" | SOR data update has been acknowledged by the UE. In this state sorXmacIue and sorMacIue shall be present; or UPU data update has been acknowledged by the UE. In this state upuXmacIue and upuMacIue shall be present; or NSSAI data update has been acknowledged by the UE or CAG data update has been acknowledged by the UE. |

### 5.4.4	Binary data

N/A

## 5.5	Error handling

# Table 5.5-1 lists common response body data structures used within the Nudr-dr API

# Table 5.5-1: Common Response Body Data Structures

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ProblemDetails | O | 0..1 | 4xx, 5xx responses | For unsuccessful status codes, the UDR may provide detailed information. |
| NOTE: In addition common data structures as defined in 3GPP TS 29.500 [8] are supported. |  |  |  |  |

The application layer error handling shall follow 3GPP TS 29.504 [2] clause 6.1.6 unless explicitly specified in this document.

## 5.6	Feature negotiation

The optional features in table 6.1.8-1 of 3GPP TS 29.504 [2] are defined for the Nudr_DataRepository API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [8].

Annex A (normative):
OpenAPI specification

## A.1	General

This Annex specifies the formal definition of the Nudr_DataRepository service API for Subscription Data. It consists of OpenAPI 3.0.0 specifications, in YAML format.

This Annex takes precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API(s).

NOTE:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification files contained in this 3GPP Technical Specification are available on a Git-based repository, that uses the GitLab software version control system (see 3GPP TS 29.501 [7] clause 5.3.1 and 3GPP TR 21.900 [14] clause 5B).

## A.2	Nudr_DataRepository API for Subscription Data

For the purpose of referencing entities in the Open API file defined in this Annex, it shall be assumed that this Open API file is contained in a physical file named "TS29505_Subscription_Data.yaml".

openapi: 3.0.0

info:

version: '-'

title: 'Unified Data Repository Service API file for subscription data'

description: |

Unified Data Repository Service (subscription data).

The API version is defined in 3GPP TS 29.504.

© 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.505 V17.2.0; 5G System; Usage of the Unified Data Repository Service for subscription data; Stage 3

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.505/'

paths:

/subscription-data/{ueId}/authentication-data/authentication-subscription:

get:

summary: Retrieves the authentication subscription data of a UE

operationId: QueryAuthSubsData

tags:

- Authentication Data (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data:authentication-subscription:read

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/AuthenticationSubscription'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: modify the authentication subscription data of a UE

operationId: ModifyAuthenticationSubscription

tags:

- Authentication Subscription (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data:authentication-subscription:modify

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

- name: supported-features

in: query

description: Features required to be supported by the target NF

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

requestBody:

content:

application/json-patch+json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

required: true

responses:

'204':

description: Expected response to a valid request

'403':

description: modification is rejected

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchResult'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/{ueId}/authentication-data/authentication-status:

put:

summary: To store the Authentication Status data of a UE

operationId: CreateAuthenticationStatus

tags:

- Authentication Status (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

requestBody:

content:

application/json:

schema:

# $ref: 'TS29503_Nudm_UEAU.yaml#/components/schemas/AuthEvent'

# required: true

responses:

'204':

description: Upon success, an empty response body shall be returned

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

get:

summary: Retrieves the Authentication Status of a UE

operationId: QueryAuthenticationStatus

tags:

- AuthEvent (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

- name: fields

in: query

description: attributes to be retrieved

required: false

schema:

type: array

items:

type: string

minItems: 1

style: form

explode: false

- name: supported-features

description: Supported Features

in: query

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: 'TS29503_Nudm_UEAU.yaml#/components/schemas/AuthEvent'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: To remove the Authentication Status of a UE

operationId: DeleteAuthenticationStatus

tags:

- AuthEvent (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

responses:

'204':

description: Upon success, an empty response body shall be returned.

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/{ueId}/authentication-data/authentication-status/{servingNetworkName}:

put:

summary: To store the individual Authentication Status data of a UE

operationId: CreateIndividualAuthenticationStatus

tags:

- Individual Authentication Status (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

- name: servingNetworkName

in: path

description: Serving Network Name

required: true

schema:

$ref: 'TS29503_Nudm_UEAU.yaml#/components/schemas/ServingNetworkName'

requestBody:

content:

application/json:

schema:

$ref: 'TS29503_Nudm_UEAU.yaml#/components/schemas/AuthEvent'

# required: true

responses:

'204':

description: Upon success, an empty response body shall be returned

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

get:

summary: Retrieves the Individual Authentication Status of a UE

operationId: QueryIndividualAuthenticationStatus

tags:

- Individual AuthEvent (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

- name: servingNetworkName

in: path

description: Serving Network Name

required: true

schema:

$ref: 'TS29503_Nudm_UEAU.yaml#/components/schemas/ServingNetworkName'

- name: fields

in: query

description: attributes to be retrieved

required: false

schema:

type: array

items:

type: string

minItems: 1

style: form

explode: false

- name: supported-features

description: Supported Features

in: query

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: 'TS29503_Nudm_UEAU.yaml#/components/schemas/AuthEvent'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: To remove the Individual Authentication Status of a UE

operationId: DeleteIndividualAuthenticationStatus

tags:

- Individual AuthEvent (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

- name: servingNetworkName

in: path

description: Serving Network Name

required: true

schema:

$ref: 'TS29503_Nudm_UEAU.yaml#/components/schemas/ServingNetworkName'

responses:

'204':

description: Upon success, an empty response body shall be returned.

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/{ueId}/ue-update-confirmation-data/sor-data:

put:

summary: To store the SoR acknowledgement information of a UE

operationId: CreateAuthenticationSoR

tags:

- Authentication SoR (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

requestBody:

content:

application/json:

schema:

# $ref: '#/components/schemas/SorData'

# required: true

responses:

'204':

description: Expected response to a valid request

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

get:

summary: Retrieves the SoR acknowledgement information of a UE

operationId: QueryAuthSoR

tags:

- Authentication SoR (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

- name: supportedFeatures

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/SorData'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/{ueId}/ue-update-confirmation-data/upu-data:

put:

summary: To store the UPU acknowledgement information of a UE

operationId: CreateAuthenticationUPU

tags:

- Authentication UPU (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

requestBody:

content:

application/json:

schema:

# $ref: '#/components/schemas/UpuData'

# required: true

responses:

'204':

description: Expected response to a valid request

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

get:

summary: Retrieves the UPU acknowledgement information of a UE

operationId: QueryAuthUPU

tags:

- Authentication UPU (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

- name: supportedFeatures

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/UpuData'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/{ueId}/ue-update-confirmation-data/subscribed-snssais:

put:

summary: To store the NSSAI update acknowledgement information of a UE

operationId: Create NSSAI update ack

tags:

- NSSAI update ack (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

requestBody:

content:

application/json:

schema:

# $ref: '#/components/schemas/NssaiAckData'

# required: true

responses:

'204':

description: Expected response to a valid request

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

get:

summary: Retrieves the UPU acknowledgement information of a UE

operationId: QueryNssaiAck

tags:

- NSSAI ACK (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

- name: supportedFeatures

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/NssaiAckData'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/{ueId}/ue-update-confirmation-data/subscribed-cag:

put:

summary: To store the CAG update acknowledgement information of a UE

operationId: CreateCagUpdateAck

tags:

- CAG update ack (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

requestBody:

content:

application/json:

schema:

# $ref: '#/components/schemas/CagAckData'

# required: true

responses:

'204':

description: Expected response to a valid request

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

get:

summary: Retrieves the CAG acknowledgement information of a UE

operationId: QueryCagAck

tags:

- CAG ACK (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/CagAckData'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/{ueId}/{servingPlmnId}/provisioned-data:

get:

summary: Retrieve multiple provisioned data sets of a UE

operationId: QueryProvisionedData

tags:

- Provisioned Data (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: servingPlmnId

in: path

description: PLMN ID

required: true

schema:

$ref: '#/components/schemas/VarPlmnId'

- name: dataset-names

in: query

description: List of dataset names

style: form

explode: false

schema:

$ref: '#/components/schemas/DatasetNames'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/ProvisionedDataSets'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/{ueId}/{servingPlmnId}/provisioned-data/am-data:

get:

summary: Retrieves the access and mobility subscription data of a UE

operationId: QueryAmData

tags:

- Access And Mobility Subscription Data (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: servingPlmnId

in: path

description: PLMN ID

required: true

schema:

$ref: '#/components/schemas/VarPlmnId'

- name: fields

in: query

description: attributes to be retrieved

required: false

schema:

type: array

items:

type: string

minItems: 1

style: form

explode: false

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: If-None-Match

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: If-Modified-Since

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.3

schema:

type: string

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/AccessAndMobilitySubscriptionData'

headers:

Cache-Control:

description: Cache-Control containing max-age, as described in RFC 7234, 5.2

schema:

type: string

ETag:

description: Entity Tag, containing a strong validator, as described in RFC 7232, 2.3

schema:

type: string

Last-Modified:

description: Timestamp for last modification of the resource, as described in RFC 7232, 2.2

schema:

type: string

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/{ueId}/{servingPlmnId}/provisioned-data/smf-selection-subscription-data:

get:

summary: Retrieves the SMF selection subscription data of a UE

operationId: QuerySmfSelectData

tags:

- SMF Selection Subscription Data (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: servingPlmnId

in: path

description: PLMN ID

required: true

schema:

$ref: '#/components/schemas/VarPlmnId'

- name: fields

in: query

description: attributes to be retrieved

required: false

schema:

type: array

items:

type: string

minItems: 1

style: form

explode: false

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: If-None-Match

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: If-Modified-Since

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.3

schema:

type: string

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/SmfSelectionSubscriptionData'

headers:

Cache-Control:

description: Cache-Control containing max-age, as described in RFC 7234, 5.2

schema:

type: string

ETag:

description: Entity Tag, containing a strong validator, as described in RFC 7232, 2.3

schema:

type: string

Last-Modified:

description: Timestamp for last modification of the resource, as described in RFC 7232, 2.2

schema:

type: string

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/{ueId}/{servingPlmnId}/provisioned-data/sm-data:

get:

summary: Retrieves the Session Management subscription data of a UE

operationId: QuerySmData

tags:

- Session Management Subscription Data

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: servingPlmnId

in: path

description: PLMN ID

required: true

schema:

$ref: '#/components/schemas/VarPlmnId'

- name: single-nssai

in: query

content:

application/json:

schema:

$ref: '#/components/schemas/VarSnssai'

description: single NSSAI

required: false

- name: dnn

in: query

description: DNN

required: false

schema:

$ref: '#/components/schemas/Dnn'

- name: fields

in: query

description: attributes to be retrieved

required: false

schema:

type: array

items:

type: string

minItems: 1

style: form

explode: false

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: If-None-Match

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: If-Modified-Since

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.3

schema:

type: string

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/SessionManagementSubscriptionData'

minItems: 1

headers:

Cache-Control:

description: Cache-Control containing max-age, as described in RFC 7234, 5.2

schema:

type: string

ETag:

description: Entity Tag, containing a strong validator, as described in RFC 7232, 2.3

schema:

type: string

Last-Modified:

description: Timestamp for last modification of the resource, as described in RFC 7232, 2.2

schema:

type: string

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/{ueId}/context-data/amf-3gpp-access:

get:

summary: Retrieves the AMF context data of a UE using 3gpp access

operationId: QueryAmfContext3gpp

tags:

- AMF 3GPP Access Registration (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: fields

in: query

description: attributes to be retrieved

required: false

schema:

type: array

items:

type: string

minItems: 1

style: form

explode: false

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/Amf3GppAccessRegistration'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

put:

summary: To store the AMF context data of a UE using 3gpp access in the UDR

operationId: CreateAmfContext3gpp

tags:

- AMF 3GPP Access Registration (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data:registrations:write

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

requestBody:

content:

application/json:

schema:

# $ref: '#/components/schemas/Amf3GppAccessRegistration'

# required: true

responses:

'204':

description: Upon success, an empty response body shall be returned

'201':

description: Created

content:

application/json:

schema:

$ref: '#/components/schemas/Amf3GppAccessRegistration'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/context-data/amf-3gpp-access'

required: true

schema:

type: string

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: To modify the AMF context data of a UE using 3gpp access in the UDR

operationId: AmfContext3gpp

tags:

- AMF 3GPP Access Registration (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data:registrations:write

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: supported-features

in: query

description: Features required to be supported by the target NF

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

requestBody:

content:

application/json-patch+json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

required: true

responses:

'204':

description: Expected response to a valid request

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchResult'

'403':

description: modification is rejected

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/{ueId}/context-data/amf-non-3gpp-access:

get:

summary: Retrieves the AMF context data of a UE using non-3gpp access

operationId: QueryAmfContextNon3gpp

tags:

- AMF Non-3GPP Access Registration (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: fields

in: query

description: attributes to be retrieved

required: false

schema:

type: array

items:

type: string

minItems: 1

style: form

explode: false

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/AmfNon3GppAccessRegistration'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

put:

summary: To store the AMF context data of a UE using non-3gpp access in the UDR

operationId: CreateAmfContextNon3gpp

tags:

- AMF Non-3GPP Access Registration (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data:registrations:write

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

requestBody:

content:

application/json:

schema:

# $ref: '#/components/schemas/AmfNon3GppAccessRegistration'

# required: true

responses:

'204':

description: Upon success, an empty response body shall be returned

'201':

description: Created

content:

application/json:

schema:

$ref: '#/components/schemas/Amf3GppAccessRegistration'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/context-data/amf-non-3gpp-access'

required: true

schema:

type: string

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: To modify the AMF context data of a UE using non 3gpp access in the UDR

operationId: AmfContextNon3gpp

tags:

- AMF Non-3GPP Access Registration (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data:registrations:write

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: supported-features

in: query

description: Features required to be supported by the target NF

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

requestBody:

content:

application/json-patch+json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

required: true

responses:

'204':

description: Expected response to a valid request

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchResult'

'403':

description: modification is rejected

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/{ueId}/context-data/smf-registrations:

get:

summary: Retrieves the SMF registration list of a UE

operationId: QuerySmfRegList

tags:

- SMF Registrations (Collection)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/SmfRegList'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/{ueId}/context-data/smf-registrations/{pduSessionId}:

get:

summary: Retrieves the individual SMF registration of a UE

operationId: QuerySmfRegistration

tags:

- SMF Registration (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: pduSessionId

in: path

description: PDU session id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionId'

- name: fields

in: query

description: attributes to be retrieved

required: false

schema:

type: array

items:

type: string

minItems: 1

style: form

explode: false

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/SmfRegistration'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

put:

summary: To create an individual SMF context data of a UE in the UDR

operationId: CreateSmfContextNon3gpp

tags:

- SMF Registration (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data:registrations:write

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: pduSessionId

in: path

description: PDU session id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionId'

requestBody:

content:

application/json:

schema:

# $ref: '#/components/schemas/SmfRegistration'

# required: true

responses:

'201':

description: Upon success, a response body containing a representation of the created Individual SmfRegistration resource shall be returned

content:

application/json:

schema:

$ref: '#/components/schemas/SmfRegistration'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/context-data/smf-registrations/{pduSessionId}'

required: true

schema:

type: string

'204':

description: No content

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: To remove an individual SMF context data of a UE the UDR

operationId: DeleteSmfContext

tags:

- SMF Registration (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data:registrations:write

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: pduSessionId

in: path

description: PDU session id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionId'

responses:

'204':

description: Upon success, an empty response body shall be returned.

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/{ueId}/operator-specific-data:

get:

summary: Retrieves the operator specific data of a UE

operationId: QueryOperSpecData

tags:

- Operator-Specific Data Container (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: fields

in: query

description: attributes to be retrieved

required: false

schema:

type: array

items:

type: string

minItems: 1

style: form

explode: false

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: If-None-Match

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: If-Modified-Since

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.3

schema:

type: string

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

type: object

description: A map(list of key-value pairs) where operator specific data element name serves as key

additionalProperties:

$ref: '#/components/schemas/OperatorSpecificDataContainer'

headers:

Cache-Control:

description: Cache-Control containing max-age, as described in RFC 7234, 5.2

schema:

type: string

ETag:

description: Entity Tag, containing a strong validator, as described in RFC 7232, 2.3

schema:

type: string

Last-Modified:

description: Timestamp for last modification of the resource, as described in RFC 7232, 2.2

schema:

type: string

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: To modify operator specific data of a UE

operationId: ModifyOperSpecData

tags:

- Operator-Specific Data Container (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: supported-features

in: query

description: Features required to be supported by the target NF

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

requestBody:

content:

application/json-patch+json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

required: true

responses:

'204':

description: Expected response to a valid request

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchResult'

'403':

description: modification is rejected

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/{ueId}/context-data/smsf-3gpp-access:

put:

summary: Create the SMSF context data of a UE via 3GPP access

operationId: CreateSmsfContext3gpp

tags:

- SMSF 3GPP Registration (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data:registrations:write

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

requestBody:

content:

application/json:

schema:

# $ref: '#/components/schemas/SmsfRegistration'

# required: true

responses:

'204':

description: Upon success, an empty response body shall be returned

'201':

description: Created

content:

application/json:

schema:

$ref: '#/components/schemas/SmsfRegistration'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/context-data/smsf-3gpp-access'

required: true

schema:

type: string

'200':

description: Updated

content:

application/json:

schema:

$ref: '#/components/schemas/SmsfRegistration'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: To remove the SMSF context data of a UE via 3GPP access

operationId: DeleteSmsfContext3gpp

tags:

- SMSF 3GPP Registration (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data:registrations:write

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

responses:

'204':

description: Upon success, an empty response body shall be returned

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

get:

summary: Retrieves the SMSF context data of a UE using 3gpp access

operationId: QuerySmsfContext3gpp

tags:

- SMSF 3GPP Registration (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: fields

in: query

description: attributes to be retrieved

required: false

schema:

type: array

items:

type: string

minItems: 1

style: form

explode: false

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/SmsfRegistration'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/{ueId}/context-data/smsf-non-3gpp-access:

put:

summary: Create the SMSF context data of a UE via non-3GPP access

operationId: CreateSmsfContextNon3gpp

tags:

- SMSF Non-3GPP Registration (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data:registrations:write

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

requestBody:

content:

application/json:

schema:

# $ref: '#/components/schemas/SmsfRegistration'

# required: true

responses:

'204':

description: Upon success, an empty response body shall be returned

'201':

description: Created

content:

application/json:

schema:

$ref: '#/components/schemas/SmsfRegistration'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/context-data/smsf-non-3gpp-access'

required: true

schema:

type: string

'200':

description: Updated

content:

application/json:

schema:

$ref: '#/components/schemas/SmsfRegistration'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: To remove the SMSF context data of a UE via non-3GPP access

operationId: DeleteSmsfContextNon3gpp

tags:

- SMSF Non-3GPP Registration (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data:registrations:write

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

responses:

'204':

description: Upon success, an empty response body shall be returned

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

get:

summary: Retrieves the SMSF context data of a UE using non-3gpp access

operationId: QuerySmsfContextNon3gpp

tags:

- SMSF Non-3GPP Registration (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: fields

in: query

description: attributes to be retrieved

required: false

schema:

type: array

items:

type: string

minItems: 1

style: form

explode: false

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/SmsfRegistration'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/{ueId}/context-data/ip-sm-gw:

put:

summary: Create the IP-SM-GW context data of a UE

operationId: CreateIpSmGwContext

tags:

- IP-SM-GW Registration (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data:registrations:write

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/IpSmGwRegistration'

# required: true

responses:

'204':

description: Upon success, an empty response body shall be returned

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: To remove the IP-SM-GW context data of a UE

operationId: DeleteIpSmGwContext

tags:

- IP-SM-GW Registration (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data:registrations:write

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

responses:

'204':

description: Upon success, an empty response body shall be returned

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: Modify the IP-SM-GW context data of a UE

operationId: ModifyIpSmGwContext

tags:

- IP-SM-GW Registration (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data:registrations:write

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

requestBody:

content:

application/json-patch+json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

minItems: 1

required: true

responses:

'204':

description: Successful response

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

get:

summary: Retrieves the IP-SM-GW context data of a UE

operationId: QueryIpSmGwContext

tags:

- IP-SM-GW Registration (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: fields

in: query

description: attributes to be retrieved

required: false

schema:

type: array

items:

type: string

minItems: 1

style: form

explode: false

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/IpSmGwRegistration'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/{ueId}/context-data/mwd:

put:

summary: Create the Message Waiting Data of the UE

operationId: CreateMessageWaitingData

tags:

- Message Waiting Data (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/MessageWaitingData'

# required: true

responses:

'204':

description: Upon success, an empty response body shall be returned

default:

description: Unexpected error

delete:

summary: To remove the Message Waiting Data of the UE

operationId: DeleteMessageWaitingData

tags:

- Message Waiting Data (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

responses:

'204':

description: Upon success, an empty response body shall be returned

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: Modify the Message Waiting Data of the UE

operationId: ModifyMessageWaitingData

tags:

- Message Waiting Data (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

requestBody:

content:

application/json-patch+json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

minItems: 1

required: true

responses:

'204':

description: Successful response

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

get:

summary: Retrieves the Message Waiting Data of the UE

operationId: QueryMessageWaitingData

tags:

- Message Waiting Data (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: fields

in: query

description: attributes to be retrieved

required: false

schema:

type: array

items:

type: string

minItems: 1

style: form

explode: false

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/MessageWaitingData'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/{ueId}/{servingPlmnId}/provisioned-data/sms-mng-data:

get:

summary: Retrieves the SMS management subscription data of a UE

operationId: QuerySmsMngData

tags:

- SMS Management Subscription Data (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: servingPlmnId

in: path

description: PLMN ID

required: true

schema:

$ref: '#/components/schemas/VarPlmnId'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: If-None-Match

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: If-Modified-Since

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.3

schema:

type: string

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/SmsManagementSubscriptionData'

headers:

Cache-Control:

description: Cache-Control containing max-age, as described in RFC 7234, 5.2

schema:

type: string

ETag:

description: Entity Tag, containing a strong validator, as described in RFC 7232, 2.3

schema:

type: string

Last-Modified:

description: Timestamp for last modification of the resource, as described in RFC 7232, 2.2

schema:

type: string

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/{ueId}/{servingPlmnId}/provisioned-data/sms-data:

get:

summary: Retrieves the SMS subscription data of a UE

operationId: QuerySmsData

tags:

- SMS Subscription Data (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: servingPlmnId

in: path

description: PLMN ID

required: true

schema:

$ref: '#/components/schemas/VarPlmnId'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: If-None-Match

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: If-Modified-Since

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.3

schema:

type: string

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/SmsSubscriptionData'

headers:

Cache-Control:

description: Cache-Control containing max-age, as described in RFC 7234, 5.2

schema:

type: string

ETag:

description: Entity Tag, containing a strong validator, as described in RFC 7232, 2.3

schema:

type: string

Last-Modified:

description: Timestamp for last modification of the resource, as described in RFC 7232, 2.2

schema:

type: string

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/{ueId}/pp-data:

get:

summary: Read the profile of a given UE

operationId: GetppData

tags:

- Parameter Provision (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: pp data for a UE

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: If-None-Match

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: If-Modified-Since

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.3

schema:

type: string

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/PpData'

headers:

Cache-Control:

description: Cache-Control containing max-age, as described in RFC 7234, 5.2

schema:

type: string

ETag:

description: Entity Tag, containing a strong validator, as described in RFC 7232, 2.3

schema:

type: string

Last-Modified:

description: Timestamp for last modification of the resource, as described in RFC 7232, 2.2

schema:

type: string

'403':

description: Forbidden

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: modify the provisioned parameter data

operationId: ModifyPpData

tags:

- ProvisionedParameterData (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: pp data for a UE

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: supported-features

in: query

description: Features required to be supported by the target NF

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

requestBody:

content:

application/json-patch+json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

required: true

responses:

'204':

description: Expected response to a valid request

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchResult'

'403':

description: modification is rejected

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/{ueId}/context-data/ee-subscriptions:

get:

summary: Retrieves the ee subscriptions of a UE

operationId: Queryeesubscriptions

tags:

- Event Exposure Subscriptions (Collection)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/EeSubscription'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

post:

summary: Create individual EE subscription

operationId: CreateEeSubscriptions

tags:

- Event Exposure Subscriptions (Collection)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE ID

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

requestBody:

content:

application/json:

schema:

# $ref: '#/components/schemas/EeSubscription'

required: true

responses:

'201':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/EeSubscription'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/context-data/ee-subscriptions/{subsId}'

required: true

schema:

type: string

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/{ueId}/context-data/ee-subscriptions/{subsId}:

put:

summary: Update an individual ee subscriptions of a UE

operationId: UpdateEesubscriptions

tags:

- Event Exposure Subscription (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: subsId

in: path

required: true

schema:

type: string

requestBody:

content:

application/json:

schema:

# $ref: '#/components/schemas/EeSubscription'

# required: true

responses:

'204':

description: Upon success, an empty response body shall be returned

'404':

description: update of non-existing resource is rejected

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Deletes a eeSubscription

operationId: RemoveeeSubscriptions

tags:

- Event Exposure Subscription (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: subsId

in: path

required: true

description: Unique ID of the subscription to remove

schema:

type: string

responses:

'204':

description: Expected response to a successful subscription removal

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: Modify an individual ee subscription of a UE

operationId: ModifyEesubscription

tags:

- Event Exposure Subscription (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: subsId

in: path

required: true

schema:

type: string

- name: supported-features

in: query

description: Features required to be supported by the target NF

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

requestBody:

content:

application/json-patch+json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

minItems: 1

required: true

responses:

'204':

description: Successful response

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchResult'

'403':

# $ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

# $ref: 'TS29571_CommonData.yaml#/components/responses/404'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

get:

summary: Retrieve a eeSubscription

operationId: QueryeeSubscription

tags:

- Event Exposure Subscription (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: subsId

in: path

required: true

description: Unique ID of the subscription to remove

schema:

type: string

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

items:

$ref: '#/components/schemas/EeSubscription'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/{ueId}/context-data/ee-subscriptions/{subsId}/amf-subscriptions:

put:

summary: Create AmfSubscriptions for an individual ee subscriptions of a UE

operationId: Create AMF Subscriptions

tags:

- AMF Subscription Info (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: subsId

in: path

required: true

schema:

type: string

requestBody:

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/AmfSubscriptionInfo'

minItems: 1

required: true

responses:

'204':

description: Upon success, an empty response body shall be returned

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Deletes AMF Subscription Info for an eeSubscription

operationId: RemoveAmfSubscriptionsInfo

tags:

- Event AMF Subscription Info (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: subsId

in: path

required: true

schema:

type: string

responses:

'204':

description: Expected response to a successful subscription removal

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: modify the AMF Subscription Info

operationId: ModifyAmfSubscriptionInfo

tags:

- AmfSubscriptionInfo (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: subsId

in: path

required: true

schema:

type: string

- name: supported-features

in: query

description: Features required to be supported by the target NF

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

requestBody:

content:

application/json-patch+json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

required: true

responses:

'204':

description: Expected response to a valid request

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchResult'

'403':

description: modification is rejected

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

get:

summary: Retrieve AMF subscription Info

operationId: GetAmfSubscriptionInfo

tags:

- Query AMF Subscription Info (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: subsId

in: path

required: true

schema:

type: string

responses:

'200':

description: OK

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/AmfSubscriptionInfo'

minItems: 1

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/{ueId}/context-data/ee-subscriptions/{subsId}/smf-subscriptions:

put:

summary: Create SMF Subscription Info

operationId: Create SMF Subscriptions

tags:

- SMF Event Subscription Info (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: subsId

in: path

required: true

schema:

type: string

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/SmfSubscriptionInfo'

required: true

responses:

'204':

description: Upon success, an empty response body shall be returned

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Delete SMF Subscription Info

operationId: RemoveSmfSubscriptionsInfo

tags:

- SMF Event Subscription Info (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: subsId

in: path

required: true

schema:

type: string

responses:

'204':

description: Expected response to a successful subscription removal

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: Modify SMF Subscription Info

operationId: ModifySmfSubscriptionInfo

tags:

- SMF Event Subscription Info (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: subsId

in: path

required: true

schema:

type: string

- name: supported-features

in: query

description: Features required to be supported by the target NF

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

requestBody:

content:

application/json-patch+json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

required: true

responses:

'204':

description: Expected response to a valid request

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchResult'

'403':

description: modification is rejected

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

get:

summary: Retrieve SMF Subscription Info

operationId: GetSmfSubscriptionInfo

tags:

- SMF Event Subscription Info (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: subsId

in: path

required: true

schema:

type: string

responses:

'200':

description: OK

content:

application/json:

schema:

$ref: '#/components/schemas/SmfSubscriptionInfo'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/group-data/{ueGroupId}/ee-subscriptions:

get:

summary: Retrieves the ee subscriptions of a group of UEs or any UE

operationId: QueryEeGroupSubscriptions

tags:

- Event Exposure Group Subscriptions (Collection)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueGroupId

in: path

description: Group of UEs or any UE

required: true

schema:

$ref: '#/components/schemas/VarUeGroupId'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/EeSubscription'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

post:

summary: Create individual EE subscription for a group of UEs or any UE

operationId: CreateEeGroupSubscriptions

tags:

- Event Exposure Group Subscriptions (Collection)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueGroupId

in: path

description: Group of UEs or any UE

required: true

schema:

$ref: '#/components/schemas/VarUeGroupId'

requestBody:

content:

application/json:

schema:

# $ref: '#/components/schemas/EeSubscription'

required: true

responses:

'201':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/EeSubscription'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/group-data/{ueGroupId}/ee-subscriptions'

required: true

schema:

type: string

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/group-data/{ueGroupId}/ee-subscriptions/{subsId}:

put:

summary: Update an individual ee subscription of a group of UEs or any UE

operationId: UpdateEeGroupSubscriptions

tags:

- Event Exposure Group Subscription (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueGroupId

in: path

required: true

schema:

$ref: '#/components/schemas/VarUeGroupId'

- name: subsId

in: path

required: true

schema:

type: string

requestBody:

content:

application/json:

schema:

# $ref: '#/components/schemas/EeSubscription'

# required: true

responses:

'204':

description: Upon success, an empty response body shall be returned

'404':

description: update of non-existing resource is rejected

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Deletes a eeSubscription for a group of UEs or any UE

operationId: RemoveEeGroupSubscriptions

tags:

- Event Exposure Group Subscription (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueGroupId

in: path

required: true

schema:

$ref: '#/components/schemas/VarUeGroupId'

- name: subsId

in: path

required: true

description: Unique ID of the subscription to remove

schema:

type: string

responses:

'204':

description: Expected response to a successful subscription removal

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: Modify an individual ee subscription for a group of a UEs

operationId: ModifyEeGroupSubscription

tags:

- Event Exposure Group Subscription (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueGroupId

in: path

required: true

schema:

$ref: '#/components/schemas/VarUeGroupId'

- name: subsId

in: path

required: true

schema:

type: string

- name: supported-features

in: query

description: Features required to be supported by the target NF

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

requestBody:

content:

application/json-patch+json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

minItems: 1

required: true

responses:

'204':

description: Successful response

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchResult'

'403':

# $ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

# $ref: 'TS29571_CommonData.yaml#/components/responses/404'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

get:

summary: Retrieve a individual eeSubscription for a group of UEs or any UE

operationId: QueryEeGroupSubscription

tags:

- Event Exposure Group Subscription (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueGroupId

in: path

required: true

schema:

$ref: '#/components/schemas/VarUeGroupId'

- name: subsId

in: path

required: true

description: Unique ID of the subscription to remove

schema:

type: string

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

items:

$ref: '#/components/schemas/EeSubscription'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/{ueId}/ee-profile-data:

get:

summary: Retrieves the ee profile data of a UE

operationId: QueryEEData

tags:

- Event Exposure Data (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: fields

in: query

description: attributes to be retrieved

required: false

schema:

type: array

items:

type: string

style: form

explode: false

- name: supportedFeatures

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/EeProfileData'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/{ueId}/context-data/sdm-subscriptions:

get:

summary: Retrieves the sdm subscriptions of a UE

operationId: Querysdmsubscriptions

tags:

- SDM Subscriptions (Collection)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/SdmSubscription'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

post:

summary: Create individual sdm subscription

operationId: CreateSdmSubscriptions

tags:

- SDM Subscriptions (Collection)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE ID

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

requestBody:

content:

application/json:

schema:

# $ref: '#/components/schemas/SdmSubscription'

required: true

responses:

'201':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/SdmSubscription'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/context-data/sdm-subscriptions/{subsId}'

required: true

schema:

type: string

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/{ueId}/context-data/sdm-subscriptions/{subsId}:

put:

summary: Update an individual sdm subscriptions of a UE

operationId: Updatesdmsubscriptions

tags:

- SDM Subscription (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: subsId

in: path

required: true

schema:

type: string

requestBody:

content:

application/json:

schema:

# $ref: '#/components/schemas/SdmSubscription'

# required: true

responses:

'204':

description: Upon success, an empty response body shall be returned

'404':

description: update of non-existing resource is rejected

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Deletes a sdmsubscriptions

operationId: RemovesdmSubscriptions

tags:

- SDM Subscription (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: subsId

in: path

required: true

description: Unique ID of the subscription to remove

schema:

type: string

responses:

'204':

description: Expected response to a successful subscription removal

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: Modify an individual sdm subscription

operationId: ModifysdmSubscription

tags:

- SDM Subscription (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: subsId

in: path

required: true

schema:

type: string

- name: supported-features

in: query

description: Features required to be supported by the target NF

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

requestBody:

content:

application/json-patch+json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

minItems: 1

required: true

responses:

'204':

description: Successful response

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchResult'

'403':

# $ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

# $ref: 'TS29571_CommonData.yaml#/components/responses/404'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

get:

summary: Retrieves a individual sdmSubscription identified by subsId

operationId: QuerysdmSubscription

tags:

- SDM Subscription (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: subsId

in: path

required: true

description: Unique ID of the subscription to retrieve

schema:

type: string

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

items:

$ref: '#/components/schemas/SdmSubscription'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/shared-data:

get:

summary: retrieve shared data

operationId: GetSharedData

tags:

- Retrieval of shared data

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: shared-data-ids

in: query

description: List of shared data ids

required: true

style: form

explode: false

schema:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/SharedDataIds'

- name: supportedFeatures

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

type: array

items:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/SharedData'

minItems: 1

'400':

# $ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

# $ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

# $ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

# $ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/subs-to-notify:

post:

summary: Subscription data subscriptions

operationId: SubscriptionDataSubscriptions

tags:

- Subs To Notify (Collection)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

requestBody:

content:

application/json:

schema:

# $ref: '#/components/schemas/SubscriptionDataSubscriptions'

required: true

responses:

'201':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/SubscriptionDataSubscriptions'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/subs-to-notify/{subsId}'

required: true

schema:

type: string

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

onDataChange:

'{request.body#/callbackReference}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/DataChangeNotify'

responses:

'204':

description: Expected response to a valid request

get:

summary: Retrieves the list of subscriptions

operationId: QuerySubsToNotify

tags:

- Subs To Notify (Collection)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ue-id

in: query

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/SubscriptionDataSubscriptions'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Deletes subscriptions identified by a given ue-id parameter

operationId: RemoveMultipleSubscriptionDataSubscriptions

tags:

- Subs To Notify (Collection)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ue-id

in: query

required: true

description: UE ID

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: nf-instance-id

in: query

required: false

description: NF Instance ID

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

- name: delete-all-nfs

in: query

required: false

description: Flag to delete subscriptions from all NFs

schema:

type: boolean

- name: implicit-unsubscribe-indication

in: query

required: false

description: Implicit Unsubscribe Indication

schema:

type: boolean

responses:

'204':

description: Expected response to a successful subscription removal

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/subs-to-notify/{subsId}:

delete:

summary: Deletes a subscriptionDataSubscriptions

operationId: RemovesubscriptionDataSubscriptions

tags:

- Subs To Notify (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: subsId

in: path

required: true

description: Unique ID of the subscription to remove

schema:

type: string

responses:

'204':

description: Expected response to a successful subscription removal

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: Modify an individual subscriptionDataSubscription

operationId: ModifysubscriptionDataSubscription

tags:

- Subs To Notify (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: subsId

in: path

required: true

schema:

type: string

- name: supported-features

in: query

description: Features required to be supported by the target NF

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

requestBody:

content:

application/json-patch+json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

minItems: 1

required: true

responses:

'204':

description: Successful response

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchResult'

'403':

# $ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

# $ref: 'TS29571_CommonData.yaml#/components/responses/404'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

get:

summary: Retrieves a individual subscriptionDataSubscription identified by subsId

operationId: QuerySubscriptionDataSubscriptions

tags:

- Subs To Notify (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: subsId

in: path

required: true

description: Unique ID of the subscription to retrieve

schema:

type: string

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

items:

$ref: '#/components/schemas/SubscriptionDataSubscriptions'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/{ueId}/{servingPlmnId}/provisioned-data/trace-data:

get:

summary: Retrieves the trace configuration data of a UE

operationId: QueryTraceData

tags:

- Trace Data (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: servingPlmnId

in: path

description: PLMN ID

required: true

schema:

$ref: '#/components/schemas/VarPlmnId'

- name: If-None-Match

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: If-Modified-Since

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.3

schema:

type: string

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TraceData'

headers:

Cache-Control:

description: Cache-Control containing max-age, as described in RFC 7234, 5.2

schema:

type: string

ETag:

description: Entity Tag, containing a strong validator, as described in RFC 7232, 2.3

schema:

type: string

Last-Modified:

description: Timestamp for last modification of the resource, as described in RFC 7232, 2.2

schema:

type: string

default:

# $ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/{ueId}/identity-data:

get:

summary: Retrieve identity data by SUPI or GPSI

operationId: GetIdentityData

tags:

- Query Identity Data by SUPI or GPSI (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE ID

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: app-port-id

in: query

description: Application port identifier

content:

application/json:

schema:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/AppPortId'

- name: If-None-Match

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: If-Modified-Since

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.3

schema:

type: string

responses:

'200':

description: OK

content:

application/json:

schema:

$ref: '#/components/schemas/IdentityData'

headers:

Cache-Control:

description: Cache-Control containing max-age, as described in RFC 7234, 5.2

schema:

type: string

ETag:

description: Entity Tag, containing a strong validator, as described in RFC 7232, 2.3

schema:

type: string

Last-Modified:

description: Timestamp for last modification of the resource, as described in RFC 7232, 2.2

schema:

type: string

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/{ueId}/operator-determined-barring-data:

get:

summary: Retrieve ODB Data data by SUPI or GPSI

operationId: GetOdbData

tags:

- Query ODB Data by SUPI or GPSI (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE ID

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

responses:

'200':

description: OK

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/OdbData'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/{ueId}/context-data:

get:

summary: Retrieve multiple context data sets of a UE

operationId: QueryContextData

tags:

- Context Data (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: context-dataset-names

in: query

description: List of context dataset names

style: form

explode: false

required: true

schema:

$ref: '#/components/schemas/ContextDatasetNames'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/ContextDataSets'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/group-data/group-identifiers:

get:

summary: Mapping of Group Identifiers

operationId: GetGroupIdentifiers

tags:

- Group Identifiers

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ext-group-id

in: query

description: External Group Identifier

required: false

schema:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/ExtGroupId'

- name: int-group-id

in: query

description: Internal Group Identifier

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

- name: ue-id-ind

in: query

description: Indication whether UE identifiers are required or not

required: false

schema:

type: boolean

default: false

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/GroupIdentifiers'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/group-data/5g-vn-groups:

get:

summary: Retrieves the data of a 5G VN Group

operationId: Query5GVnGroup

tags:

- 5G-VN-Groups (Store)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: gpsis

in: query

description: List of GPSIs

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

style: form

explode: false

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

type: object

description: A map(list of key-value pairs) where ExtGroupId serves as key

additionalProperties:

$ref: 'TS29503_Nudm_PP.yaml#/components/schemas/5GVnGroupConfiguration'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/group-data/5g-vn-groups/{externalGroupId}:

put:

summary: Create an individual 5G VN Grouop

operationId: Create5GVnGroup

tags:

- 5GVnGroupConfiguration (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: externalGroupId

in: path

required: true

schema:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/ExtGroupId'

requestBody:

content:

application/json:

schema:

# $ref: 'TS29503_Nudm_PP.yaml#/components/schemas/5GVnGroupConfiguration'

required: true

responses:

'201':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: 'TS29503_Nudm_PP.yaml#/components/schemas/5GVnGroupConfiguration'

'403':

# $ref: 'TS29571_CommonData.yaml#/components/responses/403'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Deletes the 5GVnGroup

operationId: Delete5GVnGroup

tags:

- Delete 5GVnGroup

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: externalGroupId

in: path

required: true

schema:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/ExtGroupId'

responses:

'204':

description: Expected response to a successful group removal

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: modify the 5GVnGroup

operationId: Modify5GVnGroup

tags:

- Modify 5GVnGroup

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: externalGroupId

in: path

required: true

schema:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/ExtGroupId'

- name: supported-features

in: query

description: Features required to be supported by the target NF

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

requestBody:

content:

application/json-patch+json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

required: true

responses:

'204':

description: Expected response to a valid request

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchResult'

'403':

description: modification is rejected

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

get:

summary: Retrieve a 5GVnGroup configuration

operationId: Get5GVnGroupConfiguration

tags:

- Query 5GVnGroupConfiguration (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: externalGroupId

in: path

required: true

schema:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/ExtGroupId'

responses:

'200':

description: OK

content:

application/json:

schema:

$ref: 'TS29503_Nudm_PP.yaml#/components/schemas/5GVnGroupConfiguration'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/{ueId}/lcs-privacy-data:

get:

summary: Retrieves the LCS Privacy subscription data of a UE

operationId: QueryLcsPrivacyData

tags:

- LCS Privacy Subscription Data

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: fields

in: query

description: attributes to be retrieved

required: false

schema:

type: array

items:

type: string

minItems: 1

style: form

explode: false

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: If-None-Match

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: If-Modified-Since

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.3

schema:

type: string

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/LcsPrivacyData'

headers:

Cache-Control:

description: Cache-Control containing max-age, as described in RFC 7234, 5.2

schema:

type: string

ETag:

description: Entity Tag, containing a strong validator, as described in RFC 7232, 2.3

schema:

type: string

Last-Modified:

description: Timestamp for last modification of the resource, as described in RFC 7232, 2.2

schema:

type: string

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/{ueId}/lcs-mo-data:

get:

summary: Retrieves the LCS Mobile Originated subscription data of a UE

operationId: QueryLcsMoData

tags:

- LCS Mobile Originated Subscription Data

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: fields

in: query

description: attributes to be retrieved

required: false

schema:

type: array

items:

type: string

minItems: 1

style: form

explode: false

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: If-None-Match

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: If-Modified-Since

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.3

schema:

type: string

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/LcsMoData'

headers:

Cache-Control:

description: Cache-Control containing max-age, as described in RFC 7234, 5.2

schema:

type: string

ETag:

description: Entity Tag, containing a strong validator, as described in RFC 7232, 2.3

schema:

type: string

Last-Modified:

description: Timestamp for last modification of the resource, as described in RFC 7232, 2.2

schema:

type: string

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/{ueId}/nidd-authorization-data:

get:

summary: Retrieve NIDD Authorization Data GPSI or External Group identifier

operationId: GetNiddAuData

tags:

- Query NIDD Authorization Data GPSI or External Group identifier (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE ID

required: true

schema:

type: string

pattern: '^(msisdn-[0-9]{5,15}|.+|extid-[^@]+@[^@]+|extgroupid-[^@]+@[^@]+)$'

- name: single-nssai

in: query

content:

application/json:

schema:

$ref: '#/components/schemas/VarSnssai'

description: single NSSAI

required: true

- name: dnn

in: query

description: DNN

required: true

schema:

$ref: '#/components/schemas/Dnn'

- name: mtc-provider-information

in: query

description: MTC Provider Information

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MtcProviderInformation'

- name: If-None-Match

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: If-Modified-Since

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.3

schema:

type: string

responses:

'200':

description: OK

content:

application/json:

schema:

$ref: '#/components/schemas/AuthorizationData'

headers:

Cache-Control:

description: Cache-Control containing max-age, as described in RFC 7234, 5.2

schema:

type: string

ETag:

description: Entity Tag, containing a strong validator, as described in RFC 7232, 2.3

schema:

type: string

Last-Modified:

description: Timestamp for last modification of the resource, as described in RFC 7232, 2.2

schema:

type: string

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/{ueId}/coverage-restriction-data:

get:

summary: Retrieves the subscribed enhanced Coverage Restriction Data of a UE

operationId: QueryCoverageRestrictionData

tags:

- Enhanced Coverage Restriction Data

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: If-None-Match

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: If-Modified-Since

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.3

schema:

type: string

responses:

'200':

description: OK

content:

application/json:

schema:

$ref: '#/components/schemas/EnhancedCoverageRestrictionData'

headers:

Cache-Control:

description: Cache-Control containing max-age, as described in RFC 7234, 5.2

schema:

type: string

ETag:

description: Entity Tag, containing a strong validator, as described in RFC 7232, 2.3

schema:

type: string

Last-Modified:

description: Timestamp for last modification of the resource, as described in RFC 7232, 2.2

schema:

type: string

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/{ueId}/context-data/location:

get:

summary: Retrieves the UE's Location Information

operationId: QueryUeLocation

tags:

- UE's Location Information (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/LocationInfo'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/{ueId}/v2x-data:

get:

summary: Retrieves the subscribed V2X Data of a UE

operationId: QueryV2xData

tags:

- V2X Subscription Data

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: If-None-Match

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: If-Modified-Since

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.3

schema:

type: string

responses:

'200':

description: OK

content:

application/json:

schema:

$ref: '#/components/schemas/V2xSubscriptionData'

headers:

Cache-Control:

description: Cache-Control containing max-age, as described in RFC 7234, 5.2

schema:

type: string

ETag:

description: Entity Tag, containing a strong validator, as described in RFC 7232, 2.3

schema:

type: string

Last-Modified:

description: Timestamp for last modification of the resource, as described in RFC 7232, 2.2

schema:

type: string

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscription-data/{ueId}/{servingPlmnId}/provisioned-data/lcs-bca-data:

get:

summary: Retrieves the LCS Broadcast Assistance subscription data of a UE

operationId: QueryLcsBcaData

tags:

- LCS Broadcast Assistance Subscription Data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: servingPlmnId

in: path

description: PLMN ID

required: true

schema:

$ref: '#/components/schemas/VarPlmnId'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: If-None-Match

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: If-Modified-Since

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.3

schema:

type: string

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/LcsBroadcastAssistanceTypesData'

headers:

Cache-Control:

description: Cache-Control containing max-age, as described in RFC 7234, 5.2

schema:

type: string

ETag:

description: Entity Tag, containing a strong validator, as described in RFC 7232, 2.3

schema:

type: string

Last-Modified:

description: Timestamp for last modification of the resource, as described in RFC 7232, 2.2

schema:

type: string

default:

description: Unexpected error

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

/subscription-data/group-data/5g-vn-groups/internal:

get:

summary: Retrieves the data of 5G VN Group

operationId: Query5GVnGroupInternal

tags:

- 5G-VN-Groups-Internal (Document)

parameters:

- name: internal-group-ids

in: query

description: List of Internal Group IDs

required: true

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

style: form

explode: false

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

type: object

description: A map (list of key-value pairs) where ExtGroupId serves as key

additionalProperties:

$ref: 'TS29503_Nudm_PP.yaml#/components/schemas/5GVnGroupConfiguration'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

schemas:

AuthenticationSubscription:

type: object

required:

- authenticationMethod

properties:

authenticationMethod:

# $ref: '#/components/schemas/AuthMethod'

encPermanentKey:

type: string

protectionParameterId:

type: string

sequenceNumber:

# $ref: '#/components/schemas/SequenceNumber'

authenticationManagementField:

type: string

pattern: '^[A-Fa-f0-9]{4}$'

algorithmId:

type: string

encOpcKey:

type: string

encTopcKey:

type: string

vectorGenerationInHss:

type: boolean

default: false

hssGroupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

n5gcAuthMethod:

$ref: '#/components/schemas/AuthMethod'

rgAuthenticationInd:

type: boolean

default: false

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

akmaAllowed:

type: boolean

default: false

SequenceNumber:

type: object

properties:

sqnScheme:

$ref: '#/components/schemas/SqnScheme'

sqn:

type: string

pattern: '^[A-Fa-f0-9]{12}$'

lastIndexes:

type: object

additionalProperties:

type: integer

minimum: 0

description: A map(list of key-value pairs) where type of node serves as key

indLength:

type: integer

minimum: 0

difSign:

$ref: '#/components/schemas/Sign'

SqnScheme:

anyOf:

- type: string

enum:

- GENERAL

- NON_TIME_BASED

- TIME_BASED

- type: string

Sign:

type: string

enum:

- POSITIVE

- NEGATIVE

VarPlmnId:

type: string

pattern: '^[0-9]{5,6}$'

DatasetNames:

type: array

items:

$ref: '#/components/schemas/DataSetName'

minItems: 1

uniqueItems: true

DataSetName:

anyOf:

- type: string

enum:

- AM

- SMF_SEL

- SMS_SUB

- SM

- TRACE

- SMS_MNG

- LCS_PRIVACY

- LCS_MO

- LCS_BCA

- V2X

- type: string

ProvisionedDataSets:

type: object

properties:

amData:

$ref: '#/components/schemas/AccessAndMobilitySubscriptionData'

smfSelData:

$ref: '#/components/schemas/SmfSelectionSubscriptionData'

smsSubsData:

$ref: '#/components/schemas/SmsSubscriptionData'

smData:

type: array

items:

$ref: '#/components/schemas/SessionManagementSubscriptionData'

traceData:

# $ref: 'TS29571_CommonData.yaml#/components/schemas/TraceData'

smsMngData:

# $ref: '#/components/schemas/SmsManagementSubscriptionData'

lcsPrivacyData:

# $ref: '#/components/schemas/LcsPrivacyData'

lcsMoData:

# $ref: '#/components/schemas/LcsMoData'

lcsBcaData:

# $ref: '#/components/schemas/LcsBroadcastAssistanceTypesData'

v2xData:

# $ref: '#/components/schemas/V2xSubscriptionData'

AccessAndMobilitySubscriptionData:

# $ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/AccessAndMobilitySubscriptionData'

SmfSelectionSubscriptionData:

# $ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/SmfSelectionSubscriptionData'

VarSnssai:

# $ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

Dnn:

# $ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

SessionManagementSubscriptionData:

# $ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/SessionManagementSubscriptionData'

Amf3GppAccessRegistration:

# $ref: 'TS29503_Nudm_UECM.yaml#/components/schemas/Amf3GppAccessRegistration'

AmfNon3GppAccessRegistration:

# $ref: 'TS29503_Nudm_UECM.yaml#/components/schemas/AmfNon3GppAccessRegistration'

SmfRegistration:

# $ref: 'TS29503_Nudm_UECM.yaml#/components/schemas/SmfRegistration'

SmsfRegistration:

# $ref: 'TS29503_Nudm_UECM.yaml#/components/schemas/SmsfRegistration'

LocationInfo:

# $ref: 'TS29503_Nudm_UECM.yaml#/components/schemas/LocationInfo'

SmsManagementSubscriptionData:

# $ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/SmsManagementSubscriptionData'

SmsSubscriptionData:

# $ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/SmsSubscriptionData'

LcsPrivacyData:

# $ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/LcsPrivacyData'

LcsMoData:

# $ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/LcsMoData'

AuthorizationData:

# $ref: 'TS29503_Nudm_NIDDAU.yaml#/components/schemas/AuthorizationData'

EnhancedCoverageRestrictionData:

# $ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/EnhancedCoverageRestrictionData'

V2xSubscriptionData:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/V2xSubscriptionData'

LcsBroadcastAssistanceTypesData:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/LcsBroadcastAssistanceTypesData'

OperatorSpecificDataContainer:

type: object

required:

- dataType

- value

properties:

dataType:

type: string

enum:

- string

- integer

- number

- boolean

- object

dataTypeDefinition:

type: string

value:

oneOf:

- type: string

- type: integer

- type: number

- type: boolean

- type: object

supportedFeatures:

# $ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

AuthMethod:

anyOf:

- type: string

enum:

- 5G_AKA

- EAP_AKA_PRIME

- EAP_TLS

- type: string

PpData:

# $ref: 'TS29503_Nudm_PP.yaml#/components/schemas/PpData'

EeSubscription:

# $ref: 'TS29503_Nudm_EE.yaml#/components/schemas/EeSubscription'

VarUeGroupId:

type: string

pattern: '^(extgroupid-[^@]+@[^@]+|anyUE)$'

SdmSubscription:

# $ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/SdmSubscription'

SmfRegList:

type: array

items:

$ref: '#/components/schemas/SmfRegistration'

SubscriptionDataSubscriptions:

type: object

required:

- monitoredResourceUris

- callbackReference

properties:

ueId:

# $ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

callbackReference:

# $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

originalCallbackReference:

# $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

monitoredResourceUris:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

expiry:

# $ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

sdmSubscription:

# $ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/SdmSubscription'

subscriptionId:

type: string

supported-features:

# $ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

DataChangeNotify:

type: object

properties:

originalCallbackReference:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

minItems: 1

ueId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

notifyItems:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NotifyItem'

minItems: 1

sdmSubscription:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/SdmSubscription'

# additionalSdmSubscriptions:

# type: array

# items:

# $ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/SdmSubscription'

minItems: 1

subscriptionDataSubscriptions:

type: array

items:

$ref: '#/components/schemas/SubscriptionDataSubscriptions'

IdentityData:

type: object

properties:

supiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

minItems: 1

gpsiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

minItems: 1

SorData:

type: object

properties:

provisioningTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

ueUpdateStatus:

$ref: '#/components/schemas/UeUpdateStatus'

sorXmacIue:

$ref: 'TS29509_Nausf_SoRProtection.yaml#/components/schemas/SorMac'

sorMacIue:

$ref: 'TS29509_Nausf_SoRProtection.yaml#/components/schemas/SorMac'

required:

- provisioningTime

- ueUpdateStatus

UpuData:

type: object

properties:

provisioningTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

ueUpdateStatus:

$ref: '#/components/schemas/UeUpdateStatus'

upuXmacIue:

$ref: 'TS29509_Nausf_UPUProtection.yaml#/components/schemas/UpuMac'

upuMacIue:

$ref: 'TS29509_Nausf_UPUProtection.yaml#/components/schemas/UpuMac'

required:

- provisioningTime

- ueUpdateStatus

NssaiAckData:

type: object

properties:

provisioningTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

ueUpdateStatus:

$ref: '#/components/schemas/UeUpdateStatus'

required:

- provisioningTime

- ueUpdateStatus

CagAckData:

type: object

properties:

provisioningTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

ueUpdateStatus:

$ref: '#/components/schemas/UeUpdateStatus'

required:

- provisioningTime

- ueUpdateStatus

UeUpdateStatus:

type: string

enum:

- NOT_SENT

- SENT_NO_ACK_REQUIRED

- WAITING_FOR_ACK

- ACK_RECEIVED

EeProfileData:

type: object

properties:

restrictedEventTypes:

type: array

items:

$ref: 'TS29503_Nudm_EE.yaml#/components/schemas/EventType'

supportedFeatures:

# $ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

# allowedMtcProvider:

# description: A map (list of key-value pairs where EventType serves as key) of MTC provider lists. In addition to defined EventTypes, the key value "ALL" may be used to identify a map entry which contains a list of MtcProviders that are allowed monitoring all Event Types.

# type: object

# additionalProperties:

# type: array

# items:

# $ref: '#/components/schemas/MtcProvider'

# minItems: 1

# minProperties: 1

iwkEpcRestricted:

type: boolean

default: false

imsi:

type: string

pattern: '^[0-9]{5,15}$'

hssGroupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

AmfSubscriptionInfo:

type: object

required:

- amfInstanceId

- subscriptionId

properties:

amfInstanceId:

# $ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

subscriptionId:

# $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

subsChangeNotifyCorrelationId:

type: string

contextInfo:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/ContextInfo'

ContextDatasetNames:

type: array

items:

$ref: '#/components/schemas/ContextDataSetName'

minItems: 2

uniqueItems: true

ContextDataSetName:

anyOf:

- type: string

enum:

- AMF_3GPP

- AMF_NON_3GPP

- SDM_SUBSCRIPTIONS

- EE_SUBSCRIPTIONS

- SMSF_3GPP

- SMSF_NON_3GPP

- SUBS_TO_NOTIFY

- SMF_REG

- IP_SM_GW

- type: string

ContextDataSets:

type: object

properties:

amf3Gpp:

$ref: '#/components/schemas/Amf3GppAccessRegistration'

amfNon3Gpp:

$ref: '#/components/schemas/AmfNon3GppAccessRegistration'

sdmSubscriptions:

type: array

items:

$ref: '#/components/schemas/SdmSubscription'

minItems: 1

eeSubscriptions:

type: array

items:

$ref: '#/components/schemas/EeSubscription'

minItems: 1

smsf3GppAccess:

$ref: '#/components/schemas/SmsfRegistration'

smsfNon3GppAccess:

$ref: '#/components/schemas/SmsfRegistration'

subscriptionDataSubscriptions:

type: array

items:

$ref: '#/components/schemas/SubscriptionDataSubscriptions'

minItems: 1

smfRegistrations:

$ref: '#/components/schemas/SmfRegList'

ipSmGw:

$ref: '#/components/schemas/IpSmGwRegistration'

IpSmGwRegistration:

$ref: 'TS29503_Nudm_UECM.yaml#/components/schemas/IpSmGwRegistration'

MessageWaitingData:

type: object

properties:

mwdList:

type: array

items:

$ref: '#/components/schemas/SmscData'

minItems: 1

SmscData:

type: object

anyOf:

- required: [ smscMapAddress ]

- required: [ smscDiameterAddress ]

properties:

smscMapAddress:

$ref: 'TS29503_Nudm_UECM.yaml#/components/schemas/E164Number'

smscDiameterAddress:

$ref: 'TS29503_Nudm_UECM.yaml#/components/schemas/NetworkNodeDiameterAddress'

SmfSubscriptionInfo:

description: Information related to active subscriptions at the SMF(s)

type: object

required:

- smfSubscriptionList

properties:

smfSubscriptionList:

type: array

items:

$ref: '#/components/schemas/SmfSubscriptionItem'

minItems: 1

SmfSubscriptionItem:

description: Contains info about a single SMF event subscription

type: object

required:

- smfInstanceId

- subscriptionId

properties:

smfInstanceId:

# $ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

subscriptionId:

# $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

# MtcProvider:

# type: object

# properties:

# mtcProviderInformation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MtcProviderInformation'

# afId:

# type: string

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2018-01 | CT4#82 |  |  |  |  | TS skeleton of Usage of the Unified Data Repository service for Subscription Data | 0.0.0 |
| 2018-01 | CT4#82 | C4-181397 |  |  |  | Version after CT4#82 including agreed pCRs | 0.1.0 |
| 2018-03 | CT4#83 | C4-182442 |  |  |  | Version after CT4#83 including agreed pCRs | 0.2.0 |
| 2018-04 | CT4#84 | C4-183523 |  |  |  | Version after CT4#84 including agreed pCRs | 0.3.0 |
| 2018-05 | CT4#85 | C4-184638 |  |  |  | Version after CT4#85 including agreed pCRs | 0.4.0 |
| 2018-06 | CT#80 | CP-181103 |  |  |  | Presented for information and approval | 1.0.0 |
| 2018-06 | CT#80 |  |  |  |  | Approved in CT#80. | 15.0.0 |
| 2018-09 | CT#81 | CP-182058 | 0004 | - | F | Path Segment Correction | 15.1.0 |
| 2018-09 | CT#81 | CP-182058 | 0005 | - | F | URI variables for the Resource OperatorSpecificData | 15.1.0 |
| 2018-09 | CT#81 | CP-182058 | 0008 | - | F | Formatting of query parameter | 15.1.0 |
| 2018-09 | CT#81 | CP-182212 | 0009 | 2 | B | PATCH method for ProvisionedParamenterData | 15.1.0 |
| 2018-09 | CT#81 | CP-182058 | 0010 | - | F | JSON structure in query and missing data type reference | 15.1.0 |
| 2018-09 | CT#81 | CP-182058 | 0014 | - | F | Description of Structured data types | 15.1.0 |
| 2018-09 | CT#81 | CP-182058 | 0016 | - | F | Implicit Subscriptions | 15.1.0 |
| 2018-09 | CT#81 | CP-182058 | 0011 | 1 | F | Align security and servers clause in openAPI | 15.1.0 |
| 2018-09 | CT#81 | CP-182058 | 0013 | 1 | F | VarUeId used in TS 29.505 | 15.1.0 |
| 2018-09 | CT#81 | CP-182058 | 0012 | 2 | F | OpenAPI schema for SubscriptionData | 15.1.0 |
| 2018-09 | CT#81 | CP-182058 | 0018 | 1 | F | Supported Features Negotiation | 15.1.0 |
| 2018-09 | CT#81 | CP-182041 | 0015 | 1 | B | Nudr Id Translation | 15.1.0 |
| 2018-09 | CT#81 | CP-182043 | 0001 | 2 | B | Add support for 5G Trace to Nudr | 15.1.0 |
| 2018-12 | CT#82 | CP-183016 | 0026 | - | F | Corrections in Query Parameters | 15.2.0 |
| 2018-12 | CT#82 | CP-183016 | 0030 | - | F | 29505 CR data change notification data type | 15.2.0 |
| 2018-12 | CT#82 | CP-183016 | 0021 | 1 | F | Pattern correction | 15.2.0 |
| 2018-12 | CT#82 | CP-183016 | 0022 | 1 | F | Session Management Subscription Data | 15.2.0 |
| 2018-12 | CT#82 | CP-183016 | 0019 | 1 | F | Shared Data | 15.2.0 |
| 2018-12 | CT#82 | CP-183016 | 0029 | 1 | F | 29505 CR cardinality | 15.2.0 |
| 2018-12 | CT#82 | CP-183016 | 0025 | 2 | F | Clarification of contents of SUPI list | 15.2.0 |
| 2018-12 | CT#82 | CP-183016 | 0032 | - | F | Cache control and conditional request header addition | 15.2.0 |
| 2018-12 | CT#82 | CP-183016 | 0024 | 3 | F | Introduction of Barring of Roaming in 5GC | 15.2.0 |
| 2018-12 | CT#82 | CP-183016 | 0037 | 1 | F | Session Management Subscription Data | 15.2.0 |
| 2018-12 | CT#82 | CP-183016 | 0039 | 1 | F | Authorized Event Types | 15.2.0 |
| 2018-12 | CT#82 | CP-183016 | 0040 | 1 | F | Group Subscriptions | 15.2.0 |
| 2018-12 | CT#82 | CP-183016 | 0041 | 1 | F | Location header in 201 response | 15.2.0 |
| 2018-12 | CT#82 | CP-183016 | 0038 | 2 | F | Active AmfSubscriptions storage | 15.2.0 |
| 2018-12 | CT#82 | CP-183016 | 0036 | 2 | F | lifetime of subscription | 15.2.0 |
| 2018-12 | CT#82 | CP-183158 | 0028 | 3 | F | Definition of Authentication Data for steering of roaming | 15.2.0 |
| 2018-12 | CT#82 | CP-183159 | 0027 | 4 | F | Definition of Authentication Status | 15.2.0 |
| 2018-12 | CT#82 | CP-183160 | 0020 | 5 | F | Definition of Authentication data | 15.2.0 |
| 2018-12 | CT#82 | CP-183187 | 0023 | 3 | F | Get multiple datasets for ProvisionedData | 15.2.0 |
| 2018-12 | CT#82 | CP-183249 | 0042 | - | F | Corrections on openAPI | 15.2.0 |
| 2019-03 | CT#83 | CP-190179 | 0043 |  | F | CR 0019r1 was not correctly implemented | 15.3.0 |
| 2019-03 | CT#83 | CP-190179 | 0044 |  | F | CR 0020r5 was not correctly implemented | 15.3.0 |
| 2019-03 | CT#83 | CP-190179 | 0045 |  | F | CR 0021r1 was not correctly implemented | 15.3.0 |
| 2019-03 | CT#83 | CP-190179 | 0046 |  | F | CR 0023r3 was not correctly implemented | 15.3.0 |
| 2019-03 | CT#83 | CP-190179 | 0047 |  | F | CR 0038r2 was not correctly implemented | 15.3.0 |
| 2019-03 | CT#83 | CP-190179 | 0048 |  | F | CR 0039r1 was not correctly implemented | 15.3.0 |
| 2019-03 | CT#83 | CP-190179 | 0049 |  | F | CR 0040r1 was not correctly implemented | 15.3.0 |
| 2019-03 | CT#83 | CP-190179 | 0050 |  | F | Correction of annex B | 15.3.0 |
| 2019-03 | CT#83 | CP-190179 | 0052 | 2 | F | Clarification on use of PUT | 15.3.0 |
| 2019-03 | CT#83 | CP-190179 | 0054 |  | F | Missing Headlines | 15.3.0 |
| 2019-03 | CT#83 | CP-190179 | 0055 | 2 | F | Sequence Number | 15.3.0 |
| 2019-03 | CT#83 | CP-190179 | 0056 | 1 | F | Retrieval of multiple context data sets | 15.3.0 |
| 2019-03 | CT#83 | CP-190179 | 0057 | 3 | F | UDR Subscriptions | 15.3.0 |
| 2019-03 | CT#83 | CP-190179 | 0058 | 1 | F | Data Set Names in URI Query Parameters | 15.3.0 |
| 2019-03 | CT#83 | CP-190179 | 0060 | 2 | F | AuthenticationSubscription correction | 15.3.0 |
| 2019-03 | CT#83 | CP-190179 | 0061 | 2 | F | SOR data update status | 15.3.0 |
| 2019-03 | CT#83 | CP-190168 | 0062 | 1 | F | 3GPP TS 29.505 API version update | 15.3.0 |
| 2019-06 | CT#84 | CP-191032 | 0065 |  | F | CR 0049 was not correctly implemented | 15.4.0 |
| 2019-06 | CT#84 | CP-191032 | 0066 |  | F | CR 0050 was not correctly implemented | 15.4.0 |
| 2019-06 | CT#84 | CP-191032 | 0067 |  | F | CR 0054 was not correctly implemented | 15.4.0 |
| 2019-06 | CT#84 | CP-191032 | 0068 |  | F | CR 0056r1 was not correctly implemented | 15.4.0 |
| 2019-06 | CT#84 | CP-191032 | 0070 |  | F | Update of the SubsToNotify | 15.4.0 |
| 2019-06 | CT#84 | CP-191032 | 0071 |  | F | Update of the EESubscription | 15.4.0 |
| 2019-06 | CT#84 | CP-191032 | 0064 | 1 | F | Update Individual Subscription | 15.4.0 |
| 2019-06 | CT#84 | CP-191032 | 0072 | 1 | F | missing enties of data type summary table | 15.4.0 |
| 2019-06 | CT#84 | CP-191032 | 0073 | 1 | F | correction on operationId | 15.4.0 |
| 2019-06 | CT#84 | CP-191032 | 0074 | 1 | F | 404 error response | 15.4.0 |
| 2019-06 | CT#84 | CP-191032 | 0076 | 1 | F | Correct the GET Request Body of AmfNon3GppAccessRegistration resource | 15.4.0 |
| 2019-06 | CT#84 | CP-191032 | 0077 | 1 | F | Default Notification URI | 15.4.0 |
| 2019-06 | CT#84 | CP-191032 | 0081 | 1 | F | OdbData Correction | 15.4.0 |
| 2019-06 | CT#84 | CP-191032 | 0086 | 2 | F | Storage of OpenAPI specification files | 15.4.0 |
| 2019-06 | CT#84 | CP-191032 | 0075 | 3 | F | CR 0062r1 was not correctly implemented | 15.4.0 |
| 2019-06 | CT#84 | CP-191032 | 0079 | 2 | F | Subscriptions Deletion | 15.4.0 |
| 2019-06 | CT#84 | CP-191032 | 0087 |  | F | PduSessionId | 15.4.0 |
| 2019-06 | CT#84 | CP-191032 | 0063 | 2 | F | UE Parameter Update (UPU) | 15.4.0 |
| 2019-06 | CT#84 | CP-191032 | 0078 | 2 | F | Group Identifier Translation | 15.4.0 |
| 2019-06 | CT#84 | CP-191032 | 0080 | 2 | F | Authentication Subscription | 15.4.0 |
| 2019-06 | CT#84 | CP-191032 | 0069 | 2 | F | CR 0062r1 was not correctly implemented | 15.4.0 |
| 2019-06 | CT#84 | CP-191032 | 0088 | 1 | F | Protection Parameter ID | 15.4.0 |
| 2019-06 | CT#84 | CP-191032 | 0089 | 1 | F | Copyright Note in YAML file | 15.4.0 |
| 2019-06 | CT#84 | CP-191032 | 0083 | 6 | F | SubscriptionDataSubscriptions identification | 15.4.0 |
| 2019-09 | CT#85 | CP-192229 | 0094 | 1 | B | Definition of OperatorSpecificDataContainer | 16.0.0 |
| 2019-09 | CT#85 | CP-192123 | 0100 | - | F | Missing supportedFeatures | 16.0.0 |
| 2019-09 | CT#85 | CP-192123 | 0090 | 1 | B | Network Slicing Subscription Change | 16.0.0 |
| 2019-09 | CT#85 | CP-192123 | 0098 | 1 | F | UDR Application Errors | 16.0.0 |
| 2019-09 | CT#85 | CP-192123 | 0101 | 1 | B | Application errors for DELETE method | 16.0.0 |
| 2019-09 | CT#85 | CP-192133 | 0095 | 1 | B | Closed Access Group | 16.0.0 |
| 2019-09 | CT#85 | CP-192027 | 0096 | 2 | B | VN-Group parameter provisioning | 16.0.0 |
| 2019-09 | CT#85 | CP-192120 | 0104 | - | F | 3GPP TS 29.505 externalDoc correction | 16.0.0 |
| 2019-10 |  |  |  |  |  | Corrupted references fixed | 16.0.1 |
| 2019-12 | CT#86 | CP-193029 | 0108 |  | A | Monitored Resurce URI | 16.1.0 |
| 2019-12 | CT#86 | CP-193029 | 0263 | 1 | A | GroupIdentifier resource URI | 16.1.0 |
| 2019-12 | CT#86 | CP-193049 | 0112 | 1 | B | Id translation for MSISDN-less MO SMS service | 16.1.0 |
| 2019-12 | CT#86 | CP-193278 | 0258 | 4 | B | NIDD Authorization Data | 16.1.0 |
| 2019-12 | CT#86 | CP-193131 | 0257 | 1 | B | Authentication Subscription Data | 16.1.0 |
| 2019-12 | CT#86 | CP-193055 | 0109 | 2 | B | LCS Private | 16.1.0 |
| 2019-12 | CT#86 | CP-193055 | 0110 | 3 | B | Mobile Originated Data | 16.1.0 |
| 2019-12 | CT#86 | CP-193063 | 0105 | 1 | F | DataChangeNotify | 16.1.0 |
| 2019-12 | CT#86 | CP-193063 | 0106 | 1 | B | Patch Report | 16.1.0 |
| 2019-12 | CT#86 | CP-193063 | 0259 | 1 | B | Feature Negotiation for OperatorSpecificData resource | 16.1.0 |
| 2019-12 | CT#86 | CP-193124 | 0265 | - | F | externalDocs correction | 16.1.0 |
| 2020-03 | CT#87 | CP-200018 | 0269 |  | B | Translation of Group Id to UE identifier list | 16.2.0 |
| 2020-03 | CT#87 | CP-200020 | 0271 | 1 | F | GET Method to subscriptions | 16.2.0 |
| 2020-03 | CT#87 | CP-200020 | 0272 |  | F | Add SMFRegistration to ContextDataSet | 16.2.0 |
| 2020-03 | CT#87 | CP-200020 | 0266 | 2 | F | AuthEvent deletion | 16.2.0 |
| 2020-03 | CT#87 | CP-200033 | 0267 | 1 | B | Retrieve the status of Enhanced Coverage Restriction | 16.2.0 |
| 2020-03 | CT#87 | CP-200020 | 0268 | 1 | B | 29505 CR optionality of ProblemDetails | 16.2.0 |
| 2020-03 | CT#87 | CP-200052 | 0273 |  | F | 3GPP TS 29.505 Rel16 External doc update | 16.2.0 |
| 2020-06 | CT#88-e | CP-201032 | 0275 |  | B | Location information retrieval for GMLC | 16.3.0 |
| 2020-06 | CT#88-e | CP-201032 | 0276 | 1 | B | Resource LcsPrivacySubscriptionData | 16.3.0 |
| 2020-06 | CT#88-e | CP-201071 | 0281 |  | F | Storage of YAML files in ETSI Forge | 16.3.0 |
| 2020-06 | CT#88-e | CP-201049 | 0283 |  | B | Correction on V2X Subscription data | 16.3.0 |
| 2020-06 | CT#88-e | CP-201033 | 0285 |  | B | Auth Vector Generation for HSS | 16.3.0 |
| 2020-06 | CT#88-e | CP-201034 | 0286 |  | B | Resource Level Authorization | 16.3.0 |
| 2020-06 | CT#88-e | CP-201034 | 0282 | 1 | F | Authentication results for multiple registrations | 16.3.0 |
| 2020-06 | CT#88-e | CP-201032 | 0288 | 1 | F | Mobile Originated Data retrieval | 16.3.0 |
| 2020-06 | CT#88-e | CP-201048 | 0280 | 1 | B | N5GC device Authentication | 16.3.0 |
| 2020-06 | CT#88-e | CP-201033 | 0287 | 1 | B | Support of SMSoIP | 16.3.0 |
| 2020-06 | CT#88-e | CP-201073 | 0290 |  | F | 29.505 Rel16 External doc update | 16.3.0 |
| 2020-06 | CT#88-e | CP-201021 | 0279 | 1 | A | Correction of PUT responses | 16.3.0 |
| 2020-09 | CT#89-e | CP-202110 | 0291 |  | F | Security for Authentication Subscription Data | 16.4.0 |
| 2020-09 | CT#89-e | CP-202110 | 0294 |  | F | Corrections to default responses | 16.4.0 |
| 2020-09 | CT#89-e | CP-202110 | 0295 |  | F | Corrections to the Event Group Subscription | 16.4.0 |
| 2020-09 | CT#89-e | CP-202112 | 0297 | 1 | F | Store Broadcast Location Assistance Data | 16.4.0 |
| 2020-09 | CT#89-e | CP-202110 | 0298 |  | F | Correct mismatches of definition between table and yaml | 16.4.0 |
| 2020-09 | CT#89-e | CP-202111 | 0299 |  | F | Incorrect index type | 16.4.0 |
| 2020-09 | CT#89-e | CP-202096 | 0301 |  | F | API version and External doc update | 16.4.0 |
| 2020-09 | CT#89-e | CP-202123 | 0292 |  | B | Resource Level Authorization for SubscriptionData | 17.0.0 |
| 2020-09 | CT#89-e | CP-202123 | 0293 |  | B | Resource Level Authorization for registrations | 17.0.0 |
| 2020-09 | CT#89-e | CP-202120 | 0300 |  | F | API version and External doc update | 17.0.0 |
| 2020-12 | CT#90-e | CP-203048 | 0303 |  | A | Data Removal Notification | 17.1.0 |
| 2020-12 | CT#90-e | CP-203039 | 0305 |  | A | Retrieval of SUPI from Authentication Data | 17.1.0 |
| 2020-12 | CT#90-e | CP-203048 | 0307 |  | A | Retrieve Context Data | 17.1.0 |
| 2020-12 | CT#90-e | CP-203042 | 0309 | 1 | A | 5G VN Groups | 17.1.0 |
| 2020-12 | CT#90-e | CP-203064 | 0310 |  | F | Default Notifications | 17.1.0 |
| 2020-12 | CT#90-e | CP-203039 | 0312 | 1 | A | Pattern alignment for ueId in resource URIs | 17.1.0 |
| 2020-12 | CT#90-e | CP-203035 | 0314 |  | A | Storage of YAML files in 3GPP Forge | 17.1.0 |
| 2020-12 | CT#90-e | CP-203048 | 0316 |  | A | Subs-To-Notify Tag Description | 17.1.0 |
| 2020-12 | CT#90-e | CP-203048 | 0318 |  | A | Snssai query parameter | 17.1.0 |
| 2020-12 | CT#90-e | CP-203055 | 0320 |  | F | External doc update | 17.1.0 |
| 2021-03 | CT#91-e | CP-210021 | 0321 | 1 | F | ContextInfo in AMF SubscriptionInfo | 17.2.0 |
| 2021-03 | CT#91-e | CP-210021 | 0322 | 1 | F | EE Profile Data | 17.2.0 |
| 2021-03 | CT#91-e | CP-210043 | 0324 |  | A | Missing requestBody required indication in PUT operations | 17.2.0 |
| 2021-03 | CT#91-e | CP-210021 | 0325 | 1 | F | HSS Group Id | 17.2.0 |
| 2021-03 | CT#91-e | CP-210034 | 0326 | 1 | F | Description field for map data types | 17.2.0 |
| 2021-03 | CT#91-e | CP-210048 | 0328 | 2 | A | Event Subscription Storage for SMF | 17.2.0 |
| 2021-03 | CT#91-e | CP-210021 | 0329 |  | F | Response 200 OK for SMSF registration | 17.2.0 |
| 2021-03 | CT#91-e | CP-210026 | 0330 | 1 | B | AKMA Subscription Data | 17.2.0 |
| 2021-03 | CT#91-e | CP-210029 | 0331 |  | F | 29.505 Rel-17 External doc update | 17.2.0 |