| 3GPP TS 29.537 V0.3.0 (2022-05) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; 5G System; Multicast/Broadcast Policy Control Services Stage 3 (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	7

Introduction	8

1	Scope	9

2	References	9

3	Definitions, symbols and abbreviations	10

3.1	Definitions	10

3.2	Symbols	10

3.3	Abbreviations	10

4	Overview	10

5	Services offered by the PCF	11

5.1	Introduction	11

5.2	Npcf_MBSPolicyControl Service	11

5.2.1	Service Description	11

5.2.2	Service Operations	12

5.2.2.1	Introduction	12

5.2.2.2	Npcf_MBSPoliyControl_Create	12

5.2.2.2.1	General	12

5.2.2.2.2	MBS Session Policy Association Establishment	12

5.2.2.2.3	Policy provisioning and enforcement of authorized AMBR per MBS session	13

5.2.2.3	Npcf_MBSPoliyControl_UpdateNotify	13

5.2.2.3.1	General	13

5.2.2.3.2	PCF initiated MBS Session Policy Association Update	14

5.2.2.3.3	PCF initiated MBS Session Policy Association Termination	14

5.2.2.4	Npcf_MBSPolicyControl_Delete	15

5.2.2.4.1	General	15

5.2.2.4.2	MBS Session Policy Association Deletion	15

5.2.3	MBS Policy Decision Management	15

5.2.3.1	MBS PCC rules definition	15

5.2.3.2	MBS PCC rules operation	16

5.2.3.3	MBS QoS Information	16

5.2.3.4	MBS authorized QoS	17

5.2.3.4.1	General	17

5.2.3.4.2	Policy provisioning and enforcement of MBS authorized QoS per MBS service data flow	17

5.2.3.4.3	Policy provisioning and enforcement of authorized explicitly signalled MBS QoS Characteristics	18

5.2.3.5	Provisioning and Enforcement of MBS Policy Decisions	18

5.2.3.5.1	General	18

5.2.3.5.1	MBS PCC rule	19

5.3	Npcf_MBSPolicyAuthorization Service	19

5.3.1	Service Description	19

5.3.2	Service Operations	19

5.3.2.1	Introduction	19

5.3.2.2	Npcf_MBSPoliyAuthorization_Create	20

5.3.2.2.1	General	20

5.3.2.2.2	MBS Application Session Context Establishment	20

5.3.2.3	Npcf_MBSPoliyAuthorization_Update	21

5.3.2.3.1	General	21

5.3.2.3.2	MBS Application Session Context Update	21

5.3.2.4	Npcf_MBSPoliyAuthorization_Delete	22

5.3.2.4.1	General	22

5.3.2.4.2	MBS Application Session Context Deletion	22

6	API Definitions	22

6.1	Npcf_MBSPolicyControl Service API	22

6.1.1	Introduction	22

6.1.2	Usage of HTTP	23

6.1.2.1	General	23

6.1.2.2	HTTP standard headers	23

6.1.2.2.1	General	23

6.1.2.2.2	Content type	23

6.1.2.3	HTTP custom headers	23

6.1.3	Resources	23

6.1.3.1	Overview	23

6.1.3.2	Resource: MBS Policies	24

6.1.3.2.1	Description	24

6.1.3.2.2	Resource Definition	24

6.1.3.2.3	Resource Standard Methods	24

6.1.3.2.3.1	POST	24

6.1.3.2.3.2	< method 2 >	25

6.1.3.2.4	Resource Custom Operations	25

6.1.3.2.4.1	Overview	25

6.1.3.2.4.2	Operation: < operation 1 >	25

6.1.3.2.4.3	Operation: < operation 2 >	26

6.1.3.3	Resource: Individual MBS Policy	26

6.1.3.3.1	Description	26

6.1.3.3.2	Resource Definition	26

6.1.3.3.3	Resource Standard Methods	26

6.1.3.3.3.1	GET	26

6.1.3.3.3.2	DELETE	27

6.1.4	Custom Operations without associated resources	28

6.1.4.1	Overview	28

6.1.4.2	Operation: <operation 1>	28

6.1.4.2.1	Description	29

6.1.4.2.2	Operation Definition	29

6.1.4.3	Operation: < operation 2>	29

6.1.5	Notifications	29

6.1.5.1	General	29

6.1.5.2	MBS Policy Update Notification	30

6.1.5.2.1	Description	30

6.1.5.2.2	Target URI	30

6.1.5.2.3	Standard Methods	30

6.1.5.3	MBS Policy Termination Notification	31

6.1.5.3.1	Description	31

6.1.5.3.2	Target URI	31

6.1.5.3.3	Standard Methods	32

6.1.6	Data Model	32

6.1.6.1	General	32

6.1.6.2	Structured data types	33

6.1.6.2.1	Introduction	33

6.1.6.2.2	Type: MbsPolicyCtxtData	34

6.1.6.2.3	Type: MbsPolicyDecision	34

6.1.6.2.4	Type: MbsPolicyData	35

6.1.6.2.5	Type MbsPolicyNotif	35

6.1.6.2.6	Type: MbsTermNotif	35

6.1.6.2.7	Type: MbsPccRule	36

6.1.6.2.8	Type: MbsQosInfo	36

6.1.6.2.9	Type: MbsQosChar	37

6.1.6.2.10	Type: PartialMbsSuccessReport	37

6.1.6.2.11	Type: MbsErrorReport	37

6.1.6.3	Simple data types and enumerations	37

6.1.6.3.1	Introduction	37

6.1.6.3.2	Simple data types	37

6.1.6.3.3	Enumeration: MbsPolicyAssociationReleaseCause	38

6.1.6.3.4	Enumeration: <EnumType2>	38

6.1.6.4	Data types describing alternative data types or combinations of data types	38

6.1.6.4.1	Type: <TypeName 1>	38

6.1.6.4.2	Type: <TypeName 2>	39

6.1.6.5	Binary data	39

6.1.6.5.1	Binary Data Types	39

6.1.6.5.2	< Binary Data 1 >	39

6.1.7	Error Handling	39

6.1.7.1	General	39

6.1.7.2	Protocol Errors	39

6.1.7.3	Application Errors	39

6.1.8	Feature negotiation	40

6.1.9	Security	40

6.2	Npcf_MBSPolicyAuthorization Service API	40

6.2.1	Introduction	40

6.2.2	Usage of HTTP	41

6.2.2.1	General	41

6.2.2.2	HTTP standard headers	41

6.2.2.2.1	General	41

6.2.2.2.2	Content type	41

6.2.2.3	HTTP custom headers	41

6.2.3	Resources	41

6.2.3.1	Overview	41

6.2.3.2	Resource: MBS Application Session Contexts	42

6.2.3.2.1	Description	42

6.2.3.2.2	Resource Definition	42

6.2.3.2.3	Resource Standard Methods	43

6.2.3.2.3.1	POST	43

6.2.3.2.4	Resource Custom Operations	43

6.2.3.3	Resource: Individual MBS Application Session Context	43

6.2.3.3.1	Description	43

6.2.3.3.2	Resource Definition	44

6.2.3.3.3	Resource Standard Methods	44

6.2.3.3.3.1	GET	44

6.2.3.3.3.2	PATCH	45

6.2.3.3.3.3	DELETE	46

6.2.3.3.4	Resource Custom Operations	47

6.2.4	Custom Operations without associated resources	47

6.2.5	Notifications	47

6.2.6	Data Model	47

6.2.6.1	General	47

6.2.6.2	Structured data types	47

6.2.6.2.1	Introduction	47

6.2.6.2.2	Type: MbsAppSessionCtxt	48

6.2.6.2.3	Type: MbsAppSessionCtxtPatch	48

6.2.6.3	Simple data types and enumerations	48

6.2.6.3.1	Introduction	48

6.2.6.3.2	Simple data types	48

6.2.6.3.3	Enumeration: <EnumType1>	49

6.2.6.3.4	Enumeration: <EnumType2>	49

6.2.6.4	Data types describing alternative data types or combinations of data types	49

6.2.6.4.1	Type: <TypeName 1>	49

6.2.6.4.2	Type: <TypeName 2>	49

6.2.6.5	Binary data	49

6.2.6.5.1	Binary Data Types	49

6.2.7	Error Handling	49

6.2.7.1	General	49

6.2.7.2	Protocol Errors	49

6.2.7.3	Application Errors	50

6.2.8	Feature negotiation	50

6.2.9	Security	50

Annex A (normative): OpenAPI specification	51

A.1	General	51

A.2	Npcf_MBSPolicyControl API	51

A.3	Npcf_MBSPolicyAuthorization API	56

Annex B (informative): Withdrawn API versions	60

B.1	General	60

B.2	Npcf_MBSPolicyControl API	60

B.3	Npcf_MBSPolicyAuthorization API	60

Annex C (informative): Change history	61

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

This clause is optional. If it exists, it is always the second unnumbered clause.

# 1	Scope

The present document specifies the stage 3 protocol and data model for the Service Based Interfaces of the Multicast/Broadcast Policy Control Services. It provides stage 3 protocol definitions and message flows, and specifies the APIs of the Multicast/Broadcast Services provided by the PCF.

The 5G System stage 2 architecture and procedures are specified in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3]. The stage 2 architecture and procedures for 5G Multicast/Broadcast Services are specified in 3GPP TS 23.247 [14].

The Technical Realization of the Service Based Architecture and the Principles and Guidelines for Services Definition are specified in 3GPP TS 29.500 [4] and 3GPP TS 29.501 [5].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	OpenAPI: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[7]	3GPP TR 21.900: "Technical Specification Group working methods".

[8]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[9]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[10]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[11]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[12]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[13]	IETF RFC 7807: "Problem Details for HTTP APIs".

[14]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[15]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[16]	3GPP TS 29.532: "5G System; 5G Multicast-Broadcast Session Management Services; Stage 3".

[17]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[18]	3GPP TS 29.512: "5G System; Session Management Policy Control Service; Stage 3".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

For the purpose of the present document, the terms and definitions given in clause 3 of 3GPP TS 23.247 [14] also apply, including the ones referencing other specifications.

## 3.2	Symbols

None.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

MBS	Multicast/Broadcast Service

MB-SMF	Multicast/Broadcast Session Management Function

MBSF	Multicast/Broadcast Service Function

# 4	Overview

In the frame of Multicast/Broadcast Services (MBS), the Policy Control Function (PCF) provides services to NF service consumers (e.g. MB-SMF) via the Npcf service based interface. The PCF supports for this purpose the functionalities defined in 3GPP TS 23.247 [14], i.e. MBS related Policy and Charging Control and MBS QoS control procedures.

Figures°4-1 and 4.2 depict the Multicast/Broadcast related reference architecture of the PCF respectively in SBI representation and reference point representation.

Figure 4-1: Reference model for the MBS PCF Services – SBI representation

Figure 4-2: Reference Model for the MBS PCF Services – Reference point representation

# 5	Services offered by the PCF

## 5.1	Introduction

Table 5.1-1 summarizes the corresponding APIs defined for this specification.

Table 5.1-1: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |
| Npcf_MBSPolicyControl | 5.2 | MBS Policy Control Service | TS29537_Npcf_MBSPolicyControl.yaml | npcf-mbspolicycontrol | A.2 |
| Npcf_MBSPolicyAuthorization | 5.2 | MBS Policy Authorization Service | TS29537_Npcf_MBSPolicyAuthorization.yaml | npcf-mbspolicyauth | A.3 |

## 5.2	Npcf_MBSPolicyControl Service

### 5.2.1	Service Description

The MBS Session Management Policy Control Service enables the Policy Control Function (PCF) to provision, update and remove MBS session related policies and PCC rules to NF service consumers (e.g. MB-SMF), i.e.:

-	enable NF service consumers to request the creation, update and removal of an MBS Session Policy Association;

-	enable the PCF to provision/update/remove MBS policies towards NF service consumers;

Editor's Note:	There may be updates (e.g. additional scope) depending on the progress of the related stage 2 work.

### 5.2.2	Service Operations

#### 5.2.2.1	Introduction

The service operations defined for Npcf_MBSPolicyControl are shown in table 5.2.2.1-1.

Table 5.2.2.1-1: Npcf_MBSPolicyControl Operations

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| Npcf_MBSPolicyControl_Create | Request the creation of an MBS Session Policy Association with the PCF to receive the policies for an MBS session. | NF service consumer (e.g. MB-SMF) |
| Npcf_MBSPolicyControl_UpdateNotify | Update/Provision MBS policies. | PCF |
| Npcf_MBSPolicyControl_Delete | Request the deletion of an MBS Session Policy Association. | NF service consumer (e.g. MB-SMF) |

Editor's Note:	There may be updates (e.g. additional scope) depending on the progress of the related stage 2 work.

#### 5.2.2.2	Npcf_MBSPoliyControl_Create

##### 5.2.2.2.1	General

The Npcf_MBSPolicyControl_Create service operation enablesan NF service consumer (e.g. MB-SMF) to request the creation of an MBS Session Policy Association with the PCF for a multicast or a broadcast MBS session.

The MBS Session Management procedures of the MB-SMF and related policies are defined in 3GPP TS 23.247 [x] and 3GPP TS 29.532 [16].

The following procedures using the Npcf_MBSPolicyControl_Create service operation are supported:

-	MBS Session Policy Association Establishment.

-	Provisioning of PCC rules.

-	Policy provisioning and enforcement of authorized AMBR per MBS session.

Editor's note:	The updates related to Authorized default QoS is FFS based on SA2 progress.

Editor's note: The scenarios and applicability of Session AMBR are FFS based on SA2 progress.

##### 5.2.2.2.2	MBS Session Policy Association Establishment

Figure 5.2.2.2.2-1: MBS Session Policy Association establishment

1.	In order to request the creation of an MBS Session Policy Association, the NF service consumer (e.g. MB-SMF) shall send an HTTP POST request to the PCF, as described in step 1of figure 5.2.2.2.2-1, with the request body containing the "MbsPolicyCtxtData" data structure that shall contain:

-	the concerned MBS Session Id, within the "mbsSessionId" attribute;

-	the DNN of the MBS session within the "dnn" attribute;

-	the S-NSSAI of the MBS session, within the "snssai" attribute; and

-	the URI towards which MBS policies update notifications should be sent by the PCF, within the "notificationUri" attribute.

Editor's Note:	The complete list of attributes is FFS.

2.	Upon reception of the HTTP POST request from the NF service consumer, the PCF shall perform MBS perform MBS policy authorization based on the information received from the NF service consumer and operator policies that are pre-configured at the PCF. If MBS policy authorization is successful, the PCF shall create a new "Individual MBS Policy" resource, addressed by a URI as defined in clause 6.1.3.2 and containing a PCF created resource identifier. The PCF shall then respond to the NF service consumer with an HTTP 201 Created response, including a Location header field containing the URI of the created resource and the MbsPolicyDecision data structure in the response body.

Editor's Note:	The detailed content of the MbsPolicyData data structure is FFS.

The NF service consumer shall use the URI received in the Location header in subsequent requests to the PCF to refer to the created "Individual MBS Policy" resource.

If errors occur when processing the HTTP POST request, the PCF shall apply the error handling procedures specified in clause 6.1.7.

Editor's Note:	The complete list of Error cases and the related status codes are FFS.

If the PCF, based on local configuration and/or operator policies, denies the creation of the "Individual MBS Policy" resource, the PCF may reject the request within an HTTP "403 Forbidden" status code including the "cause" attribute of the ProblemDetails data structure set to "MBS_POLICY_CONTEXT_DENIED". At reception of this error code and based on configured failure actions, the NF service consumer may reject or allow, by applying local policies, the MBS session establishment.

##### 5.2.2.2.3	Policy provisioning and enforcement of authorized AMBR per MBS session

The PCF may provision the authorized Session-AMBR to the MB-SMF in the response to the received HTTP POST message, as defined in clause 5.2.3.x.3.

Upon reception of the authorized Session-AMBR from the PCF, the MB-SMF shall apply the corresponding procedures towards the MB-UPF for the enforcement of the Session-AMBR for the concerned MBS session.

NOTE:	The authorized Session-AMBR is not propagated to the access network or the UE for the MBS session.

#### 5.2.2.3	Npcf_MBSPoliyControl_UpdateNotify

##### 5.2.2.3.1	General

The Npcf_MBSPolicyControl_UpdateNotify service operation enables the PCF to update/provision MBS Session policies to the NF service consumer (e.g. MB-SMF).

The following procedures using the Npcf_MBSPolicyControl_UpdateNotify service operation are supported:

-	PCF-initiated MBS Session Policy Association Update.

-	PCF-initiated MBS Session Policy Association Termination.

##### 5.2.2.3.2	PCF initiated MBS Session Policy Association Update

Figure 5.2.2.3.2-1: MBS Session Policy Association update

1.	The PCF may decide to provision policies related to an Individual MBS Policy resource in response to e.g. an internal trigger within the PCF. The PCF shall send for this purpose an HTTP POST request to the NF service consumer (e.g. MB-SMF) using the URI"{notificationUri}/update" with the "notificationUri" set to the notification URI received during MBS Session Policy Association establishment procedure as defined in clause 5.2.2.2. The request message body shall contain a MbsPolicyNotif data structure that shall contain:

-	the representation of the updated policies within the "mbsPolicyDecision" attribute; and

-	the identifier of the Individual MBS Policy resource related to the notification, within the "mbsPolicyId" attribute.

2.	In case of a successful update of MBS policies, a "204 No Content" response code shall be returned in the response.

If errors occur when processing the HTTP POST request, the NF service consumer shall send an HTTP error response as specified in clause 6.1.7

Editor's Note:	The complete list of Error cases and the related status codes are FFS.

If the NF service consumer received one or more MBS PCC rules from the PCF, and the validation of all these MBS PCC Rules was unsuccessful, the NF service consumer shall reject with an HTTP "400 Bad Request" status code with the response body including the MbsErrorReport data structure. Within the MbsErrorReport data structure, the NF service consumer shall include the "error" attribute containing the "cause" attribute of the ProblemDetails data structure set to "MBS_PCC_RULE_PERMANENT_FAIL" or "MBS_PCC_RULE_TEMP_FAIL" and the "mbsRuleReports" attribute to report the MBS PCC rule status of the affected MBS PCC rules.

Editor's Note:	The handling of predefined rules is FFS.

##### 5.2.2.3.3	PCF initiated MBS Session Policy Association Termination

Figure 5.2.2.3.3-1: MBS Session Policy Association termination request

1.	The PCF may request MBS session termination and the corresponding deletion of the related Individual MBS policy resource due to e.g. an internal trigger or a trigger from an AF. The PCF shall send for this purpose a notification to the NF service consumer (e.g. MB-SMF) via an HTTP POST request using the URI "{notificationUri}/terminate" with the "notificationUri" set to the notification URI received during the MBS Session Policy Association establishment procedure as defined in clause 5.2.2.2. The request body shall include the MbsTermNotif data structure that shall contain:

-	the identifier of the Individual MBS Policy association to which the notification is related, within the "mbsSessionId" attribute; and

-	the cause of why the PCF requests the termination of the MBS policy association within the "cause" attribute.

2.	If the NF service consumer accepts the received HTTP POST request, the NF service consumer shall reply with an HTTP "204 No Content" status code.

After the successful processing of the HTTP POST request, the NF service consumer shall invoke the Npcf_MBSPolicyControl_Delete Service Operation defined in clause 5.2.2.4 to terminate the corresponding "Individual MBS Policy" resource and initiate the procedure to terminate the related MBS session as defined in 3GPP TS 29.532 [16] (see also clause 7.1.1.5 of 3GPP TS 23.247 [14]).

If errors occur when processing the HTTP POST request, the NF service consumer shall send an HTTP error response as specified in clause 6.1.7.

#### 5.2.2.4	Npcf_MBSPolicyControl_Delete

##### 5.2.2.4.1	General

The Npcf_MBSPolicyControl_Delete service operation provides means for the NF service consumer (e.g. MB-SMF) to request the deletion of an existing MBS Session Policy Association.

The following procedures using the Npcf_MBSPolicyControl_Delete service operation are supported:

-	MBS Session Policy Association Deletion.

##### 5.2.2.4.2	MBS Session Policy Association Deletion

Figure 5.2.2.4.2-1: MBS Session Policy Association Deletion

1.	In order to request the deletion of an existing MBS Session Policy Association, the NF service consumer shall send an HTTP DELETE request targeting the URI of the concerned "Individual MBS Policy" resource.

2.	On successful deletion of the MBS Policy Session Association, the PCF shall respond with an HTTP "204 No Content" status code.

If errors occur when processing the HTTP DELETE request, the PCF shall apply the error handling procedures specified in clause 6.1.7.

Editor's note:	Error cases and the related MBS responses are FFS.

### 5.2.3	MBS Policy Decision Management

#### 5.2.3.1	MBS PCC rules definition

A MBS PCC rule is a set of information elements enabling the detection of a service data flow and providing parameters for policy control and/or charging control.

An MBS PCC rule consists of:

Table 5.2.3.1-1: MBS PCC rule information elements

| Information name | Description | Category |
| --- | --- | --- |
| Rule identifier | Uniquely identifies the MBS PCC rule, within a MBS Session. It is used between PCF and MB-SMF for referencing MBS PCC rules. | Mandatory |
|  | Service data flow detection |  |
| Precedence | Determines the order, in which the service data flow templates are applied at service data flow detection, enforcement and charging. | Mandatory |
| Service Data Flow Template | The list of service data flow filters for the detection of the service data flow. | Mandatory |
|  | Policy control |  |
| 5QI | Identifier of the authorized QoS parameters for the service data flow. | Mandatory |
| ARP | The Allocation and Retention Priority for the service data flow consisting of the priority level, the pre-emption capability and the pre-emption vulnerability. | Mandatory |
| MBR (DL) | The downlink maximum bitrate authorized for the service data flow. | Optional |
| GBR (DL) | The downlink guaranteed bitrate authorized for the service data flow. | Optional |
| Priority Level | Indicates a priority in scheduling resources among MBS QoS Flows. | Optional |
| Averaging Window | Represents the duration over which the guaranteed and maximum bitrate shall be calculated. | Optional |
| Maximum Data Burst Volume | Denotes the largest amount of data that is required to be transferred within a period of 5G-AN PDB. | Optional |

The exact encoding of MBS PCC rules is defined in clause 6.1.6.2.7.

#### 5.2.3.2	MBS PCC rules operation

For MBS PCC rules, the following applies:

-	Installation: to provision the MBS PCC rules.

-	Modification: to modify the MBS PCC rules.

-	Removal: to remove the MBS PCC rules.

#### 5.2.3.3	MBS QoS Information

MBS QoS info defines QoS parameters (e.g. bitrates) associated with a rule. The MBS QoS info encoding table is defined in the clause 6.1.6.2.8.

"mbsQosInfo" attribute shall include:

-	MBS QoS ID within "mbsQosId" attribute;

"mbsQosInfo" attribute may include:

-	5QI within "5qi" attribute;

-	Allocation and Retention Priority within "Arp" attribute;

-	Maximum Bit Rate for Downlink within "mbrDl" attribute;

-	Guaranteed Bit Rate Downlink within "gbrDl" attribute;

-	5QI Priority Level within "priorityLevel" attribute;

-	Averaging window within "averWindow" attribute; and

-	Maximum Data Burst Volume within "mbsMaxDataBurstVol" attribute.

#### 5.2.3.4	MBS authorized QoS

##### 5.2.3.4.1	General

The PCF shall provision the MBS authorized QoS. The MBS authorized QoS may apply to an MBS PCC rule or to an MBS session.

-	When the MBS authorized QoS applies to an MBS PCC rule, it shall be provisioned within the corresponding MBS PCC rule as defined in the clause 5.2.3.1.

-	When the MBS authorized QoS applies to a MBS session, it shall be provisioned as defined in the clause 5.2.3.x.

-	When the MBS authorized QoS applies to an explicitly signalled QoS Characteristics, it shall be provisioned as defined in the clause 5.2.3.y.

Editor's note:	Handling of Default QoS and Session AMBR applicable to MBS session is FFS.

The MBS authorized QoS provides appropriate values for the resources to be enforced. The MBS authorized QoS for a MBS PCC rule is a request for allocating the corresponding resources. The Provisioning of MBS authorized QoS per MBS PCC rule is a part of MBS PCC rule provisioning procedure.

The MB-SMF shall interact with the (R)AN (if applicable), and MB-UPF for enforcing the policy based authorization.

All the parameters of the MBS authorized QoS may be changed.

NOTE 1:	A change of 5QIs cannot be described as an upgrade or downgrade and also no 5QI can be referred to as the higher or lower. Whether the 5QI is permitted to be changed or not is subject to both operator policies and normal restrictions on changing from a non-GBR 5QI value to GBR 5QI value on an IP flow.

NOTE 2:	All attributes of the ARP QoS parameter can be changed but only the ARP priority level represents an ordered range of values. The ARP priority level attribute represents the actual priority for the service/user with the value 1 as the highest and can thus be upgraded and downgraded.

If the PCF is unable to make a decision for the response to the HTTP POST message by the MB-SMF, the PCF may reject the request as described in the clause 6.1.7.

##### 5.2.3.4.2	Policy provisioning and enforcement of MBS authorized QoS per MBS service data flow

The Provisioning of MBS authorized QoS per MBS service data flow is a part of MBS PCC rule provisioning procedure, as described in the clause 5.2.3.x.

The MBS authorized QoS per MBS service data flow shall be provisioned within a MbsQosInfo data structure. The PCF shall include a "mbsQosInfos" attribute containing the corresponding MBS QoS information decision within the MbsPolicyDecision data structure and include the reference to this MBS QoS information decision within the "refMbsQosInfo" attribute of the MbsPccRule data instance.

Within the MBS QoS Information decision, for 5QI of GBR type, the PCF shall include the authorized GBR 5QI or delay critical GBR 5QI respectively within the "5qi" attribute, the ARP within the "arp" attribute, and max bandwidth in downlink within the "mbrDl" attribute, the guaranteed bandwidth in downlink within the "gbrDl" attribute.

Within the MBS QoS information decision, for 5QI of non-GBR type, the PCF shall include the authorized non-GBR 5QI within the "5qi" attribute and the ARP within the "arp" attribute. The PCF may authorize the max bandwidth in downlink within the "mbrDl" attribute.

When the PCF authorizes a standardized 5QI but a Priority Level, an Averaging Window and/or a Maximum Data Burst Volume which are different from the standardized value in the table 5.7.4-1 of 3GPP TS 23.501 [2] are required, the PCF shall include the Priority Level within the "priorityLevel" attribute, the Averaging Window within the "averWindow" attribute and/or the Maximum Data Burst Volume within the "mbsMaxDataBurstVol" attribute.

NOTE:	For the non-standardized or non-configured 5QI, the PCF needs to authorize explicitly signalled QoS Characteristics associated with the 5QI if the PCF has not provisioned it.

The MB-SMF shall reserve the resources necessary for the guaranteed bitrate for the MBS PCC rule upon receipt of a MBS PCC rule provisioning including QoS information. For GBR QoS flows the MB-SMF should set the MBS QoS flow's GBR to the sum of the GBRs of all MBS PCC rules that are active/installed and bound to that GBR QoS flow. For GBR QoS flow the MB-SMF should set the MBS QoS flow's MBR to the sum of the MBRs of all MBS PCC rules that are active/installed and bound to that GBR QoS flow.

The MB-SMF shall assign a QFI if a new MBS QoS flow needs to be established and shall derive, if applicable, the MBS QoS profile required towards the Access Network, and the MBS QoS information with PDRs towards to the MB-UPF.

If one or more of the 5QI, ARP, Priority level, Averaging Window and Maximum Data Burst Volume attributes of a MBS PCC rule are modified to the same updated values for all the MBS PCC rules bound to the same MBS QoS flow, then the MB-SMF should modify the corresponding attributes for that impacted MBS QoS flow.

Editor's note:	Handling of pre-defined MBS PCC rules is FFS.

##### 5.2.3.4.3	Policy provisioning and enforcement of authorized explicitly signalled MBS QoS Characteristics

The PCF may provision a dynamically assigned 5QI value (from the non-standardized and non-preconfigured value range) and the associated 5G QoS characteristics to the MB-SMF. In order to do so, the PCF shall include within the MbsPolicyDecision data structure the "mbsQosChars" attribute to contain one or more authorized signalled MbsQosChar instance(s). For each MbsQosChar instance, the PCF shall include the assigned 5QI value within the "5qi" attribute, the 5QI Priority Level value within the "priorityLevel" attribute, the Packet Delay Budget value within the "packetDelayBudget" attribute, the Packet Error Rate value within the "packetErrorRate" attribute, the Averaging Window value within the "averWindow" attribute, if applicable, and the Maximum Data Burst Volume value within the "mbsMaxDataBurstVol" attribute, if applicable. If the PCF has provisioned an authorized signalled MbsQosChar instance to the MB-SMF, the PCF shall not update nor remove it during the lifetime of the policy association.

NOTE:	Operator configuration is assumed to ensure that the assigned dynamic 5QI value is unique and references the same set of QoS characteristics within the whole PLMN at a given time.

#### 5.2.3.5	Provisioning and Enforcement of MBS Policy Decisions

##### 5.2.3.5.1	General

MBS Policy Decisions are provided from the PCF to the NF service consumer (MB-SMF) as part of the following service operations:

-	the Npcf_MBSPolicyControl_Create Service Operation described in clause 5.2.2.2; and

-	the MBS Policy Association Notification request as part of the Npcf_MBSPolicyControl_UpdateNotify Service Operation as described in clause 5.2.2.3;

MBS Policy decisions shall be encoded within the MbsPolicyDecision data structure defined in clause 6.1.6.2.3

MBS Policy decisions may include:

-	MBS PCC Rule(s), as described in clause 5.2.1.x, encoded within the "mbsPccRules" attribute;

-	QoS policy decision(s), as described in clause 5.2.1.m, which can be referenced from MBS PCC rule(s), encoded within the "mbsQosInfos" attribute;

-	QoS characteristics for non-standard 5QIs and non-preconfigured 5QIs provided within the "mbsQosChars" attribute;

-	the authorized session AMBR within the "authSessAmbr" attribute

For the Npcf_MbsPolicyControl_Create Service Operation, the MbsPolicyDecision data structure shall contain a full description of all MBS policy decision(s) provided by the PCF for the MBS policy association.

For the Npcf_MBSPolicyControl_UpdateNotify service operation for the MBS Policy Association Notification request and for the Npcf_MBSPolicyControl_Update service operation, the MbsPolicyDecision data structure shall contain a description of the changes to the MBS policy decision(s) with respect to the last provided MBS policy decision(s) for the corresponding MBS policy association.

Editor's Note:	The applicable scenarios and service operations may need to be updated based on stage 2 progress.

##### 5.2.3.5.2	MBS PCC rule

The PCF may perform an operation on a single MBS PCC rule or a group of MBS PCC rules. The impacted MBS PCC rule(s) shall be included in the "mbsPccRules" map attribute within the MbsPolicyDecision data structure with the associated "pccRuleId" as the key of the map. For activating a installing or modifying a dynamic PCF-provisioned MBS PCC rule, the corresponding MbsPccRule data structure shall be provided as the map entry value. For deactivating or removing a MBS PCC rule, the map entry value shall be set to "NULL".

Editor's Note:	It is FFS is pre-defined MBS PCC rules is defined/used for MBS sessions.

In order to install a new dynamic PCF-provisioned MBS PCC rule, the PCF shall further set other attributes within the PccRule data structure as follows:

-	It may include the precedence of a MBS PCC rule among the other MBS PCC rules of the MBS session, within the "precedence" attribute. Within a MBS session, the PCF shall authorize different precedence values for the MBS PCC rules whose packet filters contained within the "flowInfos" attribute.

-	It shall include flow information within the "flowInfos" attribute.

-	It shall include one reference to the MbsQosInfo data structure within the "refMbsQosInfo" attribute. In this case, a "mbsQosInfos" attribute containing the corresponding QoS data policy decision shall be included in the MbsPolicyDecision data structure, if it has not been previously provided.

## 5.3	Npcf_MBSPolicyAuthorization Service

### 5.3.1	Service Description

The MBS Policy Authorization Service enables the Policy Control Function (PCF) to authorize the MBS application service requirements provided by an NF service consumer (e.g. AF, NEF, MBSF) and derive the related MBS policies, i.e.:

-	enable NF service consumers (e.g. AF, NEF, MBSF) to request the creation, update and removal of an MBS Application Session Context, based on the provisioning of MBS application service requirements; and

-	enable the PCF to authorize the provided MBS application service requirements and derive the policies related to the targeted MBS session.

Editor's Note:	There may be updates (e.g. additional scope, service operations) depending on the progress of the related stage 2 work in SA2.

### 5.3.2	Service Operations

#### 5.3.2.1	Introduction

The service operations defined for Npcf_MBSPolicyAuthorization are shown in table 5.3.2.1-1.

Table 5.3.2.1-1: Npcf_MBSPolicyAuthorization Service Operations

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| Npcf_MBSPolicyAuthorization_Create | Authorize the application and optionally determine and install MBS policies related to the targeted MBS Session based on the information provided by the NF service consumer. | NF service consumer (e.g. NEF, MBSF, AF) |
| Npcf_MBSPolicyAuthorization_Update | Update an existing MBS Application Session Context at the PCF and the related MBS Policy Control data based on the received updated service information. | NF service consumer (e.g. NEF, MBSF, AF) |
| Npcf_MBSPolicyAuthorization_Delete | Delete an existing MBS Application Session Context at the PCF. | NF service consumer (e.g. NEF, MBSF, AF) |

Editor's Note:	There may be updates (e.g. updated scope, service operations) depending on the progress of the related stage 2 work.

#### 5.3.2.2	Npcf_MBSPoliyAuthorization_Create

##### 5.3.2.2.1	General

The Npcf_MBSPolicyAuthorization_Create service operation enables an NF service consumer (e.g. AF, NEF, MBSF) to provision MBS application service requirements and request the PCF to authorize them and create a related MBS Application Session Context and bind it to the targeted multicast or a broadcast MBS session.

The following procedures using the Npcf_MBSPolicyAuthorization_Create service operation are supported:

-	MBS Application Session Context Establishment.

##### 5.3.2.2.2	MBS Application Session Context Establishment

Figure 5.3.2.2.2-1: MBS Application Session Context establishment

1.	In order to request the creation of an MBS Application Session Context, the NF service consumer (e.g. AF, NEF, MBSF) shall send an HTTP POST request to the PCF, as described in step 1of figure 5.3.2.2.2-1, with the request body containing the MbsAppSessionCtxt data structure that shall contain the requested MBS application service requirements.

2.	Upon reception of the HTTP POST request from the NF service consumer, the PCF shall perform MBS application service requirements authorization based on the information received from the NF service consumer and operator policies that are pre-configured at the PCF.

If the MBS application service requirements authorization is successful, the PCF shall create a new "Individual MBS Application Session Context" resource and respond to the NF service consumer with an HTTP "201 Created" status code, including an HTTP Location header field containing the URI of the created "Individual MBS Application Session Context" resource and the response body containing a representation of the created resource within the MbsAppSessionCtxt data structure.

The NF service consumer shall use the URI received in the HTTP Location header in subsequent requests to the PCF targeting the created "Individual MBS Application Session Context" resource.

If errors occur when processing the HTTP POST request, the PCF shall apply the error handling procedures specified in clause 6.2.7.

Editor's Note:	The complete list of Error cases and the related status codes are FFS.

If the service information provided in the body of the HTTP POST request is rejected because the requested MBS service is not authorized, the PCF shall reject the request with an HTTP "403 Forbidden" status code including the ProblemDetails data structure with the "cause" attribute set to "REQUESTED_MBS_SERVICE_NOT_AUTHORIZED".

If the service information provided in the body of the HTTP POST request is invalid, incorrect or insufficient for the PCF to authorize the MBS policy authorization request (e.g. invalid media type or invalid QoS reference), the PCF shall reject the request with an HTTP "400 Bad Request" status code including the ProblemDetails data structure with the "cause" attribute set to "INVALID_MBS_SERVICE_INFORMATION".

#### 5.3.2.3	Npcf_MBSPoliyAuthorization_Update

##### 5.3.2.3.1	General

The Npcf_MBSPolicyAuthorization_Update service operation enables an NF service consumer (e.g. AF, NEF, MBSF) to provision modifications to the MBS application service requirements and request the PCF to authorize them and update the associated existing MBS Application Session Context.

The following procedures using the Npcf_MBSPolicyAuthorization_Update service operation are supported:

-	MBS Application Session Context Update.

##### 5.3.2.3.2	MBS Application Session Context Update

Figure 5.3.2.3.2-1: MBS Application Session Context update

1.	In order to request the modification of an existing MBS Application Session Context, the NF service consumer (e.g. AF, NEF, MBSF) shall send an HTTP PATCH request to the PCF targeting the URI of the concerned "Individual MBS Application Session Context" resource, with the request body containing the MbsAppSessionCtxtPatch data structure that shall contain the requested modifications to the MBS application service requirements.

2.	Upon reception of the HTTP PATCH request from the NF service consumer, the PCF shall authorize the requested modifications of MBS application service requirements based on the information received from the NF service consumer and operator policies that are pre-configured at the PCF.

If the authorization of the requested modifications to the MBS application service requirements is successful, the PCF shall update the associated "Individual MBS Application Session Context" resource accordingly and respond to the NF service consumer with either an HTTP "200 OK" status code with the response body containing a representation of the updated resource within the MbsAppSessionCtxt data structure or an HTTP "204 No Content" status code.

If errors occur when processing the HTTP PATCH request, the PCF shall apply the error handling procedures specified in clause 6.2.7

Editor's Note:	Whether the HTTP PUT method should be defined instead of or in addition to the HTTP PATCH method for this service operation is FFS.

Editor's Note:	The complete list of Error cases and the related status codes are FFS.

If the updated service information provided in the body of the HTTP PATCH request is rejected because the requested MBS service is not authorized, the PCF shall reject the request with an HTTP "403 Forbidden" status code including the ProblemDetails data structure with the "cause" attribute set to "REQUESTED_MBS_SERVICE_NOT_AUTHORIZED".

#### 5.3.2.4	Npcf_MBSPoliyAuthorization_Delete

##### 5.3.2.4.1	General

The Npcf_MBSPolicyAuthorization_Delete service operation enables an NF service consumer (e.g. AF, NEF, MBSF) to request the deletion of an existing MBS Application Session Context at the PCF.

The following procedures using the Npcf_MBSPolicyAuthorization_Delete service operation are supported:

-	MBS Application Session Context Deletion.

##### 5.3.2.4.2	MBS Application Session Context Deletion

Figure 5.3.2.4.2-1: MBS Application Session Context deletion

1.	In order to request the deletion of an existing MBS Application Session Context, the NF service consumer (e.g. AF, NEF, MBSF) shall send an HTTP DELETE request to the PCF targeting the URI of the concerned "Individual MBS Application Session Context" resource.

2.	Upon success, the PCF shall respond with an HTTP "204 No Content" status code.

If errors occur when processing the HTTP DELETE request, the PCF shall apply the error handling procedures specified in clause 6.2.7

Editor's Note:	Error / redirection cases and the related status codes are FFS.

# 6	API Definitions

## 6.1	Npcf_MBSPolicyControl Service API

### 6.1.1	Introduction

The Npcf_MBSPolicyControl shall use the Npcf_MBSPolicyControl API.

The API URI of the Npcf_MBSPolicyControl API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests from the NF service consumer towards the NF service producer shall have the Resource URI structure defined in clause 4.4.1 of 3GPP TS 29.501 [5], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [5].

-	The <apiName> shall be "npcf-mbspolicycontrol".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 6.1.3.

### 6.1.2	Usage of HTTP

#### 6.1.2.1	General

HTTP/2, IETF RFC 7540 [11], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

The OpenAPI [6] specification of HTTP messages and content bodies for the Npcf_MBSPolicyControl API is contained in Annex A.2.

#### 6.1.2.2	HTTP standard headers

##### 6.1.2.2.1	General

See clause 5.2.2 of 3GPP TS 29.500 [4] for the usage of HTTP standard headers.

##### 6.1.2.2.2	Content type

JSON, IETF RFC 8259 [12], shall be used as content type of the HTTP bodies specified in the present specification as specified in clause 5.4 of 3GPP TS 29.500 [4]. The use of the JSON format shall be signalled by the content type "application/json".

"Problem Details" JSON object shall be used to indicate additional details of the error in a HTTP response body and shall be signalled by the content type "application/problem+json", as defined in IETF RFC 7807 [13].

#### 6.1.2.3	HTTP custom headers

The mandatory HTTP custom header fields specified in clause 5.2.3.2 of 3GPP TS 29.500 [4] shall be supported, and the optional HTTP custom header fields specified in clause 5.2.3.3 of 3GPP TS 29.500 [4] may be supported.

### 6.1.3	Resources

#### 6.1.3.1	Overview

This clause describes the structure for the resource URIs and the resources and methods used for the Npcf_MBSPolicyControl service.

Figure 6.1.3.1-1 depicts the resource URIs structure for the Npcf_MBSPolicyControl API.

Figure 6.1.3.1-1: Resource URI structure of the Npcf_MBSPolicyControl API

Table 6.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.1.3.1-1: Resources and methods overview

| Resource purpose/name | Resource URI (relative path after API URI) | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| MBS Policies | /mbs-policies | POST | Request the creation of a new Individual MBS Policy resource. |
| Individual MBS Policy | /mbs-policies/{mbsPolicyId} | GET | Request the retrieval of an Individual MBS Policy resource. |
|  |  | DELETE | Request the deletion of an Individual MBS Policy resource. |

#### 6.1.3.2	Resource: MBS Policies

##### 6.1.3.2.1	Description

This resource represents the collection of Individual MBS Policy resources managed by the PCF.

##### 6.1.3.2.2	Resource Definition

Resource URI: {apiRoot}/npcf-mbspolicycontrol/<apiVersion>/mbs-policies

This resource shall support the resource URI variables defined in table 6.1.3.2.2-1.

Table 6.1.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1. |

##### 6.1.3.2.3	Resource Standard Methods

###### 6.1.3.2.3.1	POST

This method shall support the URI query parameters specified in table 6.1.3.2.3.1-1.

Table 6.1.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.3.2.3.1-3.

Table 6.1.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MbsPolicyCtxtData | M | 1 | Contains the parameters to create an individual MBS Policy resource. |

Table 6.1.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MbsPolicyDecision | M | 1 | 201 Created | Successful case. An Individual MBS Policy resource is successfully created and a representation of the created resource is returned to the NF service consumer. |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |
| NOTE 1: The manadatory HTTP error status code for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: Failure cases are described in clause 6.1.7. |  |  |  |  |

Table 6.1.3.2.3.1-4: Headers supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/npcf-mbspolicycontrol/<apiVersion>/mbs-policies/{mbsPolicyId} |

###### 6.1.3.2.3.2	< method 2 >

And so on if there are more than two methods supported by the resource. Same structure as in clause 6.1.3.2.3.1.

##### 6.1.3.2.4	Resource Custom Operations

The following clauses will specify the custom operations supported by the resource.

It will describe, for each custom operation, the use and the URI of the operation, the HTTP method on which it is mapped, request and response data structures and response codes, and if applicable, HTTP headers specific to the operation.

###### 6.1.3.2.4.1	Overview

Table 6.1.3.2.4.1-1: Custom operations

| Operation name | Custom operaration URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| <custom operation name> | <custom operation URI> | e.g.POST | <Operation executed by Custom operation> |

###### 6.1.3.2.4.2	Operation: < operation 1 >

This clause will specify the meaning of the operation applied on the resource.

6.1.3.2.4.2.1	Description

This sublause will describe the custom operation and what it is used for, and the custom operation's URI.

6.1.3.2.4.2.2	Operation Definition

This clause will specify the custom operation and the HTTP method on which it is mapped.

This operation shall support the request data structures specified in table 6.1.3.2.4.2.2-1 and the response data structure and response codes specified in table 6.1.3.2.4.2.2-2.

Table 6.1.3.2.4.2.2-1: Data structures supported by the <e.g. POST> Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1", or "M..N", or <leave empty> | <only if applicable> |

Table 6.1.3.2.4.2.2-2: Data structures supported by the <e.g. POST> Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1" or "M..N", or <leave empty> | <list applicable codes with name from the applicable RFCs> | <Meaning of the success case> or <Meaning of the error case with additional statement regarding error handling> |
| NOTE: The manadatory HTTP error status code for the <e.g. POST> method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

###### 6.1.3.2.4.3	Operation: < operation 2 >

And so on if there are more than two operations supported by the resource. Same structure as in clause 6.1.3.2.4.1.

#### 6.1.3.3	Resource: Individual MBS Policy

##### 6.1.3.3.1	Description

This resource represents an Individual MBS Policy resource managed by the PCF.

##### 6.1.3.3.2	Resource Definition

Resource URI: {apiRoot}/npcf-mbspolicycontrol/<apiVersion>/mbs-policies/{mbsPolicyId}

This resource shall support the resource URI variables defined in table 6.1.3.3.2-1.

Table 6.1.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1. |
| mbsPolicyId | string | Represents the unique identifier of the individual MBS Policy resource. |

##### 6.1.3.3.3	Resource Standard Methods

###### 6.1.3.3.3.1	GET

This method shall support the URI query parameters specified in table 6.1.3.3.3.1-1.

Table 6.1.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.1-2 and the response data structures and response codes specified in table 6.1.3.3.3.1-3.

Table 6.1.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MbsPolicyData | M | 1 | 200 OK | Successful case. The requested Individual MBS Policy resource is successfully returned to the NF service consumer. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative PCF (service) instance. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative PCF (service) instance. |
| NOTE: The manadatory HTTP error status code for the GET method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Editor's Note:	Error / redirection cases and the related status codes are FFS.

Table 6.1.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative PCF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected |

Table 6.1.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative PCF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected |

###### 6.1.3.3.3.2	DELETE

This method enables an NF service consumer (e.g. MB-SMF) to request the deletion of an existing "Individual MBS Policy" resource at the PCF.

This method shall support the URI query parameters specified in table 6.1.3.3.3.2-1.

Table 6.1.3.3.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.2-2 and the response data structures and response codes specified in table 6.1.3.3.3.2-3.

Table 6.1.3.3.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.3.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response Codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful response. The Individual MBS Policy resource was successfully deleted. |
|  |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative PCF (service) instance. |
|  |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative PCF (service) instance. |
| NOTE 1: The mandadatory HTTP error status code for the DELETE method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Editor's Note:	Errors are FFS.

Table 6.1.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative PCF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected |

Table 6.1.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative PCF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected |

### 6.1.4	Custom Operations without associated resources

#### 6.1.4.1	Overview

This clause will specify custom operations without any associated resource (i.e. RPC) supported by this API.

Table 6.1.4.1-1: Custom operations without associated resources

| Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- |
| <custom operation URI> | e.g.POST | <Operation executed by Custom operation> |

#### 6.1.4.2	Operation: <operation 1>

Where <operation 1> is to be replaced by the name of the custom operation, e.g. Authentication_Information_Request.

It will describe, for each custom operation, the use and the URI of the operation, the HTTP method on which it is mapped, request and response data structures and response codes, and if applicable, HTTP headers specific to the operation.

##### 6.1.4.2.1	Description

This sublause will describe the custom operation and what it is used for, and the custom operation's URI.

##### 6.1.4.2.2	Operation Definition

This clause will specify the custom operation and the HTTP method on which it is mapped.

This operation shall support the response data structures and response codes specified in tables 6.1.4.2.2-1 and 6.1.4.2.2-2.

Table 6.1.4.2.2-1: Data structures supported by the <e.g. POST> Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1", or "M..N", or <leave empty> | <only if applicable> |

Table 6.1.4.2.2-2: Data structures supported by the <e.g. POST> Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "M", "C" or "O" | "0..1", "1" or "M..N", or <leave empty> | <list applicable codes with name from the applicable RFCs> | <Meaning of the success case> or <Meaning of the error case with additional statement regarding error handling> |
| NOTE: The manadatory HTTP error status code for the <e.g. POST> method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

#### 6.1.4.3	Operation: < operation 2>

And so on if there are more than one custom operations supported by the service. Same structure as in clause 6.1.4.2.

### 6.1.5	Notifications

#### 6.1.5.1	General

Notifications shall comply to clause 6.2 of 3GPP TS 29.500 [4] and clause 4.6.2.3 of 3GPP TS 29.501 [5].

Table 6.1.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| MBS Policy Update Notification | {notificationUri}/update | update (POST) | MBS Policy Update Notification initiated by the PCF. |
| MBS Policy Termination Notification | {notificationUri}/terminate | terminate (POST) | MBS Policy Termination Notification initiated by the PCF. |

#### 6.1.5.2	MBS Policy Update Notification

##### 6.1.5.2.1	Description

This notification is used by the PCF to provision/update of MBS session policies to the NF service consumer (e.g. MB-SMF).

Editor's Note:	There may be updates (e.g. additional scope) depending on the progress of the related stage 2 work

##### 6.1.5.2.2	Target URI

The Callback URI "{notificationUri}" shall be used with the callback URI variables defined in table 6.1.5.2.2-1.

Table 6.1.5.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notificationUri | String formatted as URI with the Callback Uri |

##### 6.1.5.2.3	Standard Methods

6.1.5.2.3.1	POST

This method shall support the request data structures specified in table 6.1.5.2.3.1-1 and the response data structures and response codes specified in table 6.1.5.2.3.1-2.

Table 6.1.5.2.3.1-1: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MbsPolicyNotif | M | 1 | Provision/Update of MBS policies by the PCF. |

Table 6.1.5.2.3.1-2: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The MBS policies are provisioned/updated successfully. |
| array(PartialMbsSuccessReport) | O | 1..N | 200 OK | Some of the MBS PCC rules provisioned by the PCF are not installed successfully. |
| MbsErrorReport | M | 1 | 400 Bad Request | The MBS policies including all the MBS PCC rules provisioned by the PCF are not installed successfully. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative NF service consumer (service) instance where the notification should be sent. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative NF service consumer (service) instance where the notification should be sent. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Editor's Note:	The handling of predefined rules is FFS.

Table 6.1.5.2.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative NF consumer (service) instance towards which the notification should be redirected. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the notification request is redirected |

Table 6.1.5.2.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative NF consumer (service) instance towards which the notification should be redirected. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the notification request is redirected |

Editor's Note:	There may be updates (e.g. additional scope) depending on the progress of the related stage 2 work.

#### 6.1.5.3	MBS Policy Termination Notification

##### 6.1.5.3.1	Description

This notification is used by the PCF to request the termination of an existing MBS policy to the NF service consumer (e.g. MB-SMF).

##### 6.1.5.3.2	Target URI

The Callback URI "{notificationUri}/terminate" shall be used with the callback URI variables defined in table 6.1.5.3.2-1.

Table 6.1.5.3.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notificationUri | String formatted as URI with the Callback Uri. |

##### 6.1.5.3.3	Standard Methods

6.1.5.3.3.1	POST

This method shall support the request data structures specified in table 6.1.5.3.3.1-1 and the response data structures and response codes specified in table 6.1.5.3.3.1-2.

Table 6.1.5.3.3.1-1: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MbsTermNotif | M | 1 | Request to terminate the MBS policy association. |

Table 6.1.5.3.3.1-2: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The MBS policy termination request is successfully acknowledged. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative NF service consumer (service) instance where the notification should be sent. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative NF service consumer (service) instance where the notification should be sent. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 6.1.5.3.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative NF consumer (service) instance towards which the notification should be redirected. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the notification request is redirected. |

Table 6.1.5.3.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative NF consumer (service) instance towards which the notification should be redirected. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the notification request is redirected. |

### 6.1.6	Data Model

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Npcf_MBSPolicyControl service based interface protocol.

Table 6.1.6.1-1: Npcf_MBSPolicyControl specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| MbsPccRule | 6.1.6.2.7 | Represents an MBS PCC rule's information. |  |
| MbsPolicyCtxtData | 6.1.6.2.2 | Contains the parameters used to request the creation of an Individual MBS Policy resource. |  |
| MbsPolicyData | 6.1.6.2.4 | Contains the MBS policy data of an Individual MBS Policy resource. |  |
| MbsPolicyDecision | 6.1.6.2.3 | Contains the MBS policies authorized by the PCF. |  |
| MbsPolicyNotif | 6.1.6.2.5 | Represents an MBS policy update notification. |  |
| MbsQosChar | 6.1.6.2.9 | Represents explicitly signalled QoS characteristics. |  |
| MbsQosInfo | 6.1.6.2.8 | Represents MBS QoS information. |  |
| MbsTermNotif | 6.1.6.2.6 | Represents an MBS policy termination notification. |  |
| PartialMbsSuccessReport | 6.1.6.2.10 | Includes the information reported by the NF service consumer when some of the MBS PCC rules are not successfully installed. |  |
| MbsErrorReport | 6.1.6.2.11 | Contains the rule reports. |  |

Table 6.1.6.1-2 specifies data types re-used by the Npcf_MBSPolicyControl service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Npcf_MBSPolicyControl service based interface.

Table 6.1.6.1-2: Npcf_MBSPolicyControl re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| 5Qi | 3GPP TS 29.571 [15] | Indicates the 5G QoS Identifier. |  |
| 5QiPriorityLevel | 3GPP TS 29.571 [15] | Indicates the 5QI Priority Level. |  |
| Arp | 3GPP TS 29.571 [15] | Indicates the allocation and retention priority. |  |
| AverWindow | 3GPP TS 29.571 [15] | Indicates the Averaging Window. |  |
| BitRate | 3GPP TS 29.571 [15] | Indicates the Bit Rate. |  |
| Dnn | 3GPP TS 29.571 [15] | Identifies a DNN. |  |
| FlowDescription | 3GPP TS 29.512 [18] | Represents packet filter for an IP flow. |  |
| MbsSessionId | 3GPP TS 29.571 [15] | Represents an MBS Session Identifier. |  |
| PacketDelBudget | TS 29.571 [15] | Indicates Packet Delay Budget. |  |
| PacketErrRate | TS 29.571 [15] | Indicates Packet Error Rate. |  |
| RedirectResponse | 3GPP TS 29.571 [15] | Contains redirection related information. |  |
| Snssai | 3GPP TS 29.571 [15] | Identifies an S-NSSAI. |  |
| SupportedFeatures | 3GPP TS 29.571 [15] | Represents the list of supported features. It is used to negotiate the applicability of the optional features. |  |
| Uri | 3GPP TS 29.571 [15] | Represents a URI. |  |
| Uinteger | 3GPP TS 29.571 [15] | Represents an unsigned integer |  |

Editor's note:	Whether the data type MbsMaxDataBurstVol will be defined in 3GPP TS 29.537 or 3GPP TS 29.571 is FFS.

#### 6.1.6.2	Structured data types

##### 6.1.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 6.1.6.2.2	Type: MbsPolicyCtxtData

Table 6.1.6.2.2-1: Definition of type MbsPolicyCtxtData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mbsSessionId | MbsSessionId | M | 1 | Represents the identifier of the concerned MBS Session. |  |
| dnn | Dnn | M | 1 | Represents the DNN of the MBS session. |  |
| snssai | Snssai | M | 1 | Represents the S-NSSAI of the MBS session. |  |
| notificationUri | Uri | M | 1 | Represents the URI towards which MBS policies update notifications are sent by the PCF. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of the features defined in clause 6.1.8. This parameter shall be provided if at least one feature is supported by the NF Service Consumer. |  |

Editor's Note:	The list of attributes and conditions are FFS and pending stage 2 progress.

##### 6.1.6.2.3	Type: MbsPolicyDecision

Table 6.1.6.2.3-1: Definition of type MbsPolicyDecision

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mbsPccRules | map(MbsPccRule) | O | 1..N | A map of MBS PCC rules with the content being the MbsPccRule as described in clause 6.1.6.2.7. The key used in this map for each entry is the pccRuleId attribute of the corresponding MbsPccRule. |  |
| mbsQosInfos | map(MbsQosInfo) | O | 1..N | Map of QoS info policy decisions. The key used in this map for each entry is the qosId attribute of the corresponding MbsQosInfo. (NOTE) |  |
| mbsQosChars | map(MbsQosChar) | O | 1..N | Map of QoS characteristics for non-standard 5QIs and non-preconfigured 5QIs. This map uses the 5QI values as keys. (NOTE) |  |
| authSessAmbr (FFS) | Ambr | O | 0..1 | Authorized Session-AMBR. |  |
| NOTE: This attribute shall not be removed if it was provisioned. |  |  |  |  |  |

Editor's Note:	 It is FFS how to convey Session AMBR and other possible MBS Policy information at session level.

Editor's Note:	The list of attributes is FFS.

##### 6.1.6.2.4	Type: MbsPolicyData

Table 6.1.6.2.4-1: Definition of type MbsPolicyData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mbsPolicyCtxtData | MbsPolicyCtxtData | M | 0..1 | Contains the parameters used to request the MBS policy creation. |  |
| mbsPolicy | MbsPolicyDecision | O | 0..1 | Contains the MBS policies authorized by the PCF. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of negotiated supported features. This parameter shall be provided by the PCF in the response to a request in which the NF service consumer provided the list of features that it supports. |  |

Editor's Note:	The list of attributes and conditions are FFS and pending stage 2 progress.

##### 6.1.6.2.5	Type MbsPolicyNotif

Table 6.1.6.2.5-1: Definition of type MbsPolicyNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mbsSessionId | string | M | 1 | The identifier of the Individual MBS Policy resource to which the notification is related. |  |
| mbsPolicyDecision | MbsPolicyDecision | O | 1 | MBS Session management policy decision (see clause 6.1.6.2.3). |  |

##### 6.1.6.2.6	Type: MbsTermNotif

Table 6.1.6.2.6-1: Definition of type MbsTermNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mbsSessionId | string | M | 1 | The identifier of an Individual MBS Policy association to which the notification is related. |  |
| cause | MbsPolicyAssociationReleaseCause | M | 1 | The cause why the PCF requests the termination of the policy association. |  |

##### 6.1.6.2.7	Type: MbsPccRule

Table 6.1.6.2.7-1: Definition of type MbsPccRule

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dlIpFlowInfos | array(FlowDescription) | O | 1..N | An array of downlink IP flow packet filter information. |  |
| pccRuleId | string | M | 1 | Univocally identifies the PCC rule within a MBS session. |  |
| precedence | Uinteger | O | 0..1 | Determines the order in which this PCC rule is applied relative to other PCC rules within the same MBS session. (NOTE 1) |  |
| refMbsQosInfo | array(string) | O | 1..N | A reference to the MbsQosInfo policy decision type. (NOTE 2) |  |
| NOTE 1: The "precedence" attribute is used to specify the precedence of the PCC rule among all PCC rules associated with the MBS session. It includes an integer value in the range from 0 to 255 (decimal). The higher the value of the "precedence" attribute, the lower the precedence of that PCC rule is. NOTE 2: Arrays are only introduced for future compatibility. In this release of the specification the maximum number of elements in the array is 1. |  |  |  |  |  |

##### 6.1.6.2.8	Type: MbsQosInfo

Table 6.1.6.2.8-1: Definition of type MbsQosInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mbsQosId | string | M | 1 | Univocally identifies the QoS control policy data within a MBS session. |  |
| 5qi | 5Qi | O | 0..1 | Identifier for the authorized QoS parameters for the service data flow. |  |
| mbrDl | BitRate | O | 0..1 | Indicates the maximum bandwidth in downlink. |  |
| gbrDl | BitRate | O | 0..1 | Indicates the guaranteed bandwidth in downlink. (NOTE) |  |
| arp | Arp | O | 0..1 | Indicates the allocation and retention priority. |  |
| priorityLevel | 5QiPriorityLevel | O | 0..1 | Indicates a priority in scheduling resources among QoS Flows. (NOTE) |  |
| averWindow | AverWindow | O | 0..1 | Represents the duration over which the guaranteed and maximum bitrates shall be calculated. (NOTE) |  |
| mbsMaxDataBurstVol | MbsMaxDataBurstVol | O | 0..1 | Denotes the largest amount of data that is required to be transferred within a period. (NOTE) |  |
| NOTE: Applicable only when a value different from the standardized value for this 5QI, provided in table 5.7.4-1 3GPP TS 23.501 [2], is required. |  |  |  |  |  |

##### 6.1.6.2.9	Type: MbsQosChar

Table 6.1.6.2.9-1: Definition of type MbsQosChar

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| 5qi | 5Qi | M | 1 | Identifier for the MBS authorized QoS parameters for the MBS service data flow. Applies to MBS PCC rule and MBS session level. |  |
| resourceType | QosResourceType | M | 1 | Indicates whether the resource type is GBR, delay critical GBR, or non-GBR. |  |
| priorityLevel | 5QiPriorityLevel | M | 1 | Unsigned integer indicating the 5QI Priority Level, within a range of 1 to 127. |  |
| packetDelayBudget | PacketDelBudget | M | 1 | Unsigned integer indicates the packet delay budget. Packet Delay Budget expressed in milliseconds. |  |
| packetErrorRate | PacketErrRate | M | 1 | String indicating the packet error rate. Examples: Packer Error Rate 4x10-6 shall be encoded as "4E-6". Packer Error Rate 10-2 shall be encoded as"1E-2". |  |
| averWindow | AverWindow | C | 0..1 | Indicates the averaging window. This IE shall be present only for a GBR QoS flow or a Delay Critical GBR QoS flow. |  |
| mbsMaxDataBurstVol | MbsMaxDataBurstVol | M | 1 | Unsigned Integer. Indicates the maximum data burst volume. |  |

##### 6.1.6.2.10	Type: PartialMbsSuccessReport

Table 6.1.6.2.10-1: Definition of type PartialMbsSuccessReport

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |

Editor's Note:	The complete list of attributes is FFS.

##### 6.1.6.2.11	Type: MbsErrorReport

Table 6.1.6.2.11-1: Definition of type MbsErrorReport

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |

Editor's Note:	The complete list of attributes is FFS.

#### 6.1.6.3	Simple data types and enumerations

##### 6.1.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 6.1.6.3.2	Simple data types

The simple data types defined in table 6.1.6.3.2-1 shall be supported.

Table 6.1.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
|  | <one simple data type, i.e. boolean, integer, number, or string> |  |  |

##### 6.1.6.3.3	Enumeration: MbsPolicyAssociationReleaseCause

The enumeration MbsPolicyAssociationReleaseCause represents the cause why the PCF requests the termination of the policy association. It shall comply with the provisions defined in table 6.1.6.3.3-1.

Table 6.1.6.3.3-1: Enumeration MbsPolicyAssociationReleaseCause

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| UNSPECIFIED | This value is used for unspecified reasons. |  |

Editor's note:	Other cause values are FFS.

##### 6.1.6.3.4	Enumeration: <EnumType2>

And so on if there are more enumerations to define.

#### 6.1.6.4	Data types describing alternative data types or combinations of data types

##### 6.1.6.4.1	Type: <TypeName 1>

The data types describing alternative data types or combinations of data types shall represent an OpenAPI schema object using the "oneOf", "anyOf" or "allOf"  keyword to list alternative or to be combined data types (see the OpenAPI specification [4] and https://swagger.io/docs/specification/data-models/oneof-anyof-allof-not/).

An instance (i.e. a corresponding part of a JSON file to be evaluated against the schema) matches , i.e. a list of mutually exclusive alternative data types., as described using the OpenAPI "oneOf" keyword, if the instance matches against one and only one of the alternative data types.

An instance (i.e. a corresponding part of a JSON file to be evaluated against the schema) matches a list of non-exclusive alternative data types, as described using the OpenAPI "anyOf" keyword, if the instance matches against one or more of the alternative data types.

An instance (i.e. a corresponding part of a JSON file to be evaluated against the schema) matches a list of to be combined data types, as described using the OpenAPI "allOf" keyword, if the instance matches against all of the to be combined data types."Attribute name": Name of attributes that belong to the specified data type. The attribute names within a structured data type shall be unique, and their relative order inside the structured data type shall not imply any specific ordering of the corresponding JSON elements in a JSON object.

"Data type": Data type of the alternative or to be combined data type. If the data type is indicated as "<type>", the alternative or to be combined data type shall be of data type <type>. If the data type is indicated as "array(<type>)", the alternative or to be combined data type shall be an array (see IETF RFC 8259 [3]) that contains elements of data type <type>. If the data type is indicated as "map(<type>)", the alternative or to be combined data type shall be an object (see IETF RFC 8259 [3]) encoded in the corresponding OpenAPI specification as a map which values are of data type <type>. <type> can either be "integer", "number", "string" or "boolean" (as defined in the OpenAPI specification [4]), or a data type defined in a 3GPP specification.

"Cardinality": Defines the allowed number of occurrence of data type <type>. A cardinality of "M..N", is only allowed for data types "array(<type>)" and "map(<type>)" and indicates the number of elements within the array or map; the values M and N can either be the characters "M" and "N", respectively, or integer numbers; with M being greater than or equal 0, and N being greater than 0. For data type "<type>", the cardinality shall be set to "1".

"Description": Describes the meaning and use of the attribute and may contain normative statements.Applicability: If the attribute is only applicable for optional feature(s) negotiated using the mechanism defined in clause 6.6 of 3GPP TS 29.500 [4], the name of the corresponding feature(s) shall be indicated in this column. If no feature is indicated. the attribute can be used with any feature.

Applicability: If the type is only applicable for optional feature(s) negotiated using the mechanism defined in clause 6.6 of 3GPP TS 29.500 [2], the name of the corresponding feature(s) shall be indicated in this column. If no feature is indicated. the type can be used with any feature. If no optional features are defined for an API, the applicability column can be omitted for that API.

Table 6.1.6.4.1-1: Definition of type <Type name 1> as a list of <"mutually exclusive alternatives" / "non-exclusive alternatives" / "to be combined data types">

| Data type | Cardinality | Description | Applicability |
| --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "1" or "M..N" | <only if applicable> |  |

##### 6.1.6.4.2	Type: <TypeName 2>

And so on if there are more types to specify.

#### 6.1.6.5	Binary data

##### 6.1.6.5.1	Binary Data Types

Table 6.1.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |
| < Binary Data 1 > e.g. N1 SM Message | < clause > e.g. 6.1.6.5.2 | <content type> e.g. vnd.3gpp.5gnas |

##### 6.1.6.5.2	< Binary Data 1 >

And so on if there are more binary data to specify

### 6.1.7	Error Handling

#### 6.1.7.1	General

For the Npcf_MBSPolicyControl API, HTTP error responses shall be supported as specified in clause 4.8 of 3GPP TS 29.501 [5]. Protocol errors and application errors specified in table 5.2.7.2-1 of 3GPP TS 29.500 [4] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.2.7.1-1 of 3GPP TS 29.500 [4].

In addition, the requirements in the following clauses are applicable for the Npcf_MBSPolicyControl API.

#### 6.1.7.2	Protocol Errors

No specific procedures for the Npcf_MBSPolicyControl service are specified.

#### 6.1.7.3	Application Errors

The application errors defined for the Npcf_MBSPolicyControl service are listed in Table 6.1.7.3-1.

Table 6.1.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| MBS_PCC_RULE_PERMANENT_FAIL | 400 Bad Request | The HTTP request is rejected because all the MBS PCC rules provisioned by the PCF in the request cannot be installed. It is used to inform the PCF that the request failed and should not be attempted again. |
| MBS_PCC_RULE_TEMP_FAIL | 400 Bad Request | The HTTP request is rejected because for some reason all the MBS PCC rules provisioned by the PCF in the request cannot be enforced or modified successfully in a network initiated procedure. It is used to inform the PCF that the request could not be satisfied at the time it was received but may be able to satisfy the request in the future. |
| MBS_POLICY_CONTEXT_DENIED | 403 Forbidden | The HTTP request is rejected because the PCF does not accept the NF service consumer request due to operator policies and/or local configuration. |

Editor's Note:	The complete list of application errors is FFS.

### 6.1.8	Feature negotiation

The optional features in table 6.1.8-1 are defined for the Npcf_MBSPolicyControl API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 6.1.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 6.1.9	Security

As indicated in 3GPP TS 33.501 [8] and 3GPP TS 29.500 [4], the access to the Npcf_MBSPolicyControl API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [9]), based on local configuration, using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [10]) plays the role of the authorization server.

If OAuth2 is used, an NF Service Consumer, prior to consuming services offered by the Npcf_MBSPolicyControl API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [10], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Npcf_MBSPolicyControl service.

The Npcf_MBSPolicyControl API defines a single scope "mbspolicycontrol" for the entire service, and it does not define any additional scopes at resource or operation level.

## 6.2	Npcf_MBSPolicyAuthorization Service API

### 6.2.1	Introduction

The Npcf_MBSPolicyAuthorization shall use the Npcf_MBSPolicyAuthorization API.

The API URI of the Npcf_MBSPolicyAuthorization API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests from the NF service consumer towards the NF service producer shall have the Resource URI structure defined in clause 4.4.1 of 3GPP TS 29.501 [5], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [5].

-	The <apiName> shall be "npcf-mbspolicyauth".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 6.2.3.

### 6.2.2	Usage of HTTP

#### 6.2.2.1	General

HTTP/2, IETF RFC 7540 [11], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

The OpenAPI [6] specification of HTTP messages and content bodies for the Npcf_MBSPolicyAuthorization API is contained in Annex A.2.

#### 6.2.2.2	HTTP standard headers

##### 6.2.2.2.1	General

See clause 5.2.2 of 3GPP TS 29.500 [4] for the usage of HTTP standard headers.

##### 6.2.2.2.2	Content type

JSON, IETF RFC 8259 [12], shall be used as content type of the HTTP bodies specified in the present specification as specified in clause 5.4 of 3GPP TS 29.500 [4]. The use of the JSON format shall be signalled by the content type "application/json".

"Problem Details" JSON object shall be used to indicate additional details of the error in a HTTP response body and shall be signalled by the content type "application/problem+json", as defined in IETF RFC 7807 [13].

#### 6.2.2.3	HTTP custom headers

The mandatory HTTP custom header fields specified in clause 5.2.3.2 of 3GPP TS 29.500 [4] shall be supported, and the optional HTTP custom header fields specified in clause 5.2.3.3 of 3GPP TS 29.500 [4] may be supported.

### 6.2.3	Resources

#### 6.2.3.1	Overview

This clause describes the structure of the resource URIs and the resources and methods used for the Npcf_MBSPolicyAuthorization service.

Figure 6.2.3.1-1 depicts the resource URIs structure for the Npcf_MBSPolicyAuthorization API.

Figure 6.2.3.1-1: Resource URI structure of the Npcf_MBSPolicyAuthorization API

Table 6.2.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.2.3.1-1: Resources and methods overview

| Resource purpose/name | Resource URI (relative path after API URI) | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| MBS Application Session Contexts | /mbs-app-sessions | POST | Create a new Individual MBS Application Session Context resource. |
| Individual MBS Application Session Context | /mbs-app-sessions/{mbsAppSessionId} | GET | Retrieve an existing Individual MBS Application Session Context resource. |
|  |  | PATCH | Modify an existing Individual MBS Application Session Context resource. |
|  |  | DELETE | Delete an existing Individual MBS Application Session Context resource. |

Editor's Note:	Whether the HTTP PUT method should be defined instead of or in addition to the HTTP PATCH method is FFS.

#### 6.2.3.2	Resource: MBS Application Session Contexts

##### 6.2.3.2.1	Description

This resource represents the collection of Individual MBS Application Session Context resources managed by the PCF.

##### 6.2.3.2.2	Resource Definition

Resource URI: {apiRoot}/npcf-mbspolicyauth/<apiVersion>/mbs-app-sessions

This resource shall support the resource URI variables defined in table 6.2.3.2.2-1.

Table 6.2.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.2.1. |

##### 6.2.3.2.3	Resource Standard Methods

###### 6.2.3.2.3.1	POST

This method shall support the URI query parameters specified in table 6.2.3.2.3.1-1.

Table 6.2.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.2.3.1-2 and the response data structures and response codes specified in table 6.2.3.2.3.1-3.

Table 6.2.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MbsAppSessionCtxt | M | 1 | Contains the parameters to create an Individual MBS Application Session Context resource. |

Table 6.2.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MbsAppSessionCtxt | M | 1 | 201 Created | Successful case. An Individual MBS Application Session Context resource is successfully created and a representation of the created resource is returned to the NF service consumer. |
| ProblemDetails | O | 0..1 | 400 Bad Request | (NOTE 2) |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status code for the HTTP POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: Failure cases are described in clause 6.2.7. |  |  |  |  |

Table 6.2.3.2.3.1-4: Headers supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/npcf-mbspolicyauth/<apiVersion>/mbs-app-sessions/{mbsAppSessionId} |

##### 6.2.3.2.4	Resource Custom Operations

There are no resource custom operations defined under this resource in this release of the specification.

#### 6.2.3.3	Resource: Individual MBS Application Session Context

##### 6.2.3.3.1	Description

This resource represents an Individual MBS Application Session Context resource managed by the PCF.

##### 6.2.3.3.2	Resource Definition

Resource URI: {apiRoot}/npcf-mbspolicyauth/<apiVersion>/mbs-app-sessions/{mbsAppSessionId}

This resource shall support the resource URI variables defined in table 6.2.3.3.2-1.

Table 6.2.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.2.1. |
| mbsAppSessionId | string | Represents the unique identifier of the Individual MBS Application Session Context resource. |

##### 6.2.3.3.3	Resource Standard Methods

###### 6.2.3.3.3.1	GET

This method shall support the URI query parameters specified in table 6.2.3.3.3.1-1.

Table 6.2.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.3.3.1-2 and the response data structures and response codes specified in table 6.2.3.3.3.1-3.

Table 6.2.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MbsAppSessionCtxt | M | 1 | 200 OK | Successful case. The requested Individual MBS Application Session Context resource is successfully returned to the NF service consumer. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative PCF (service) instance. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative PCF (service) instance. |
| NOTE: The mandatory HTTP error status code for the HTTP GET method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Editor's Note:	Error / redirection cases and the related status codes are FFS.

Table 6.2.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative PCF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

Table 6.2.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative PCF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

###### 6.2.3.3.3.2	PATCH

This method shall support the URI query parameters specified in table 6.2.3.3.3.2-1.

Table 6.2.3.3.3.2-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.3.3.2-2 and the response data structures and response codes specified in table 6.2.3.3.3.2-3.

Table 6.2.3.3.3.2-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MbsAppSessionCtxtPatch | M | 1 | Contains the parameters to request the modification of an existing Individual MBS Application Session Context resource. |

Table 6.2.3.3.3.2-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MbsAppSessionCtxt | M | 1 | 200 OK | Successful case. The concerned Individual MBS Application Session Context resource is successfully modified and a representation of the updated resource is returned to the NF service consumer in the response body. |
| n/a |  |  | 204 No Content | Successful case. The concerned Individual MBS Application Session Context resource is successfully modified and no content is returned in the response body. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative PCF (service) instance. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative PCF (service) instance. |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status code for the HTTP PATCH method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: Failure cases are described in clause 6.2.7. |  |  |  |  |

Table 6.2.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative PCF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

Table 6.2.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative PCF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

###### 6.2.3.3.3.3	DELETE

This method shall support the URI query parameters specified in table 6.2.3.3.3.3-1.

Table 6.2.3.3.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.3.3.3-2 and the response data structures and response codes specified in table 6.2.3.3.3.3-3.

Table 6.2.3.3.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.3.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The concerned Individual MBS Application Session Context resource is successfully deleted. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative PCF (service) instance. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative PCF (service) instance. |
| NOTE: The mandatory HTTP error status code for the HTTP DELETE method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Editor's Note:	Error / redirection cases and the related status codes are FFS.

Table 6.2.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative PCF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

Table 6.2.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative PCF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected. |

##### 6.2.3.3.4	Resource Custom Operations

There are no resource custom operations defined under this resource in this release of the specification.

### 6.2.4	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

### 6.2.5	Notifications

There are no notifications defined for this API in this release of the specification.

### 6.2.6	Data Model

#### 6.2.6.1	General

This clause specifies the application data model supported by the API.

Table 6.2.6.1-1 specifies the data types defined for the Npcf_MBSPolicyAuthorization service based interface protocol.

Table 6.2.6.1-1: Npcf_MBSPolicyControl specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| MbsAppSessionCtxt | 6.2.6.2.2 | Identifies the service requirements of an Individual MBS Application Session Context. |  |
| MbsAppSessionCtxtPatch | 6.2.6.2.3 | Describes the modifications to an existing Individual MBS Application Session Context resource. |  |

Table 6.2.6.1-2 specifies data types re-used by the Npcf_MBSPolicyControl service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Npcf_MBSPolicyControl service based interface.

Table 6.2.6.1-2: Npcf_MBSPolicyControl re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Dnn | 3GPP TS 29.571 [15] | Identifies a DNN. |  |
| MbsSessionId | 3GPP TS 29.571 [15] | Represents an MBS Session Identifier. |  |
| RedirectResponse | 3GPP TS 29.571 [15] | Contains redirection related information. |  |
| Snssai | 3GPP TS 29.571 [15] | Identifies an S-NSSAI. |  |
| SupportedFeatures | 3GPP TS 29.571 [15] | Represents the list of supported features. It is used to negotiate the applicability of the optional features. |  |

#### 6.2.6.2	Structured data types

##### 6.2.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 6.2.6.2.2	Type: MbsAppSessionCtxt

Table 6.2.6.2.2-1: Definition of type MbsAppSessionCtxt

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mbsSessionId | MbsSessionId | M | 1 | Represents the identifier of the concerned MBS Session. |  |
| dnn | Dnn | C | 0..1 | Represents the DNN of the MBS session. This attribute shall be provided, if available. |  |
| snssai | Snssai | C | 0..1 | Represents the S-NSSAI of the MBS session. This attribute shall be provided, if available. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of the supported features (among the ones defined in clause 6.2.8). This parameter shall be provided if at least one feature is supported by the NF Service Consumer. |  |

Editor's Note:	The final and full list of attributes and the related conditions are still FFS and pending stage 2 progress.

Editor's Note:	Whether more than one application ID can be provided and how it is handled is FFS.

##### 6.2.6.2.3	Type: MbsAppSessionCtxtPatch

Table 6.2.6.2.3-1: Definition of type MbsAppSessionCtxtPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |

Editor's Note:	The list of attributes and the related conditions are still FFS and pending stage 2 progress.

#### 6.2.6.3	Simple data types and enumerations

##### 6.2.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 6.2.6.3.2	Simple data types

The simple data types defined in table 6.2.6.3.2-1 shall be supported.

Table 6.2.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

##### 6.2.6.3.3	Enumeration: <EnumType1>

The enumeration <EnumType1> represents <something>. It shall comply with the provisions defined in table 6.2.6.3.3-1.

Table 6.2.6.3.3-1: Enumeration < EnumType1>

| Enumeration value | Description | Applicability |
| --- | --- | --- |

##### 6.2.6.3.4	Enumeration: <EnumType2>

And so on if there are more enumerations to define.

#### 6.2.6.4	Data types describing alternative data types or combinations of data types

##### 6.2.6.4.1	Type: <TypeName 1>

Table 6.2.6.4.1-1: Definition of type <Type name 1> as a list of <"mutually exclusive alternatives" / "non-exclusive alternatives" / "to be combined data types">

| Data type | Cardinality | Description | Applicability |
| --- | --- | --- | --- |
| "<type>" or "array(<type>)" or "map(<type>)" | "1" or "M..N" | <only if applicable> |  |

##### 6.2.6.4.2	Type: <TypeName 2>

And so on if there are more types to specify.

#### 6.2.6.5	Binary data

##### 6.2.6.5.1	Binary Data Types

Table 6.2.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

### 6.2.7	Error Handling

#### 6.2.7.1	General

For the Npcf_MBSPolicyAuthorization API, HTTP error responses shall be supported as specified in clause 4.8 of 3GPP TS 29.501 [5]. Protocol errors and application errors specified in table 5.2.7.2-1 of 3GPP TS 29.500 [4] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.2.7.1-1 of 3GPP TS 29.500 [4].

In addition, the requirements in the following clauses are applicable for the Npcf_MBSPolicyAuthorization API.

#### 6.2.7.2	Protocol Errors

No specific procedures for the Npcf_MBSPolicyAuthorization service are specified.

#### 6.2.7.3	Application Errors

The application errors defined for the Npcf_MBSPolicyAuthorization service are listed in table 6.2.7.3-1.

Table 6.2.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| INVALID_MBS_SERVICE_INFORMATION | 400 Bad Request | The HTTP request is rejected because the service information is invalid or insufficient for the PCF to perform the requested action, e.g. invalid media type or invalid QoS reference. |
| REQUESTED_MBS_SERVICE_NOT_AUTHORIZED | 403 Forbidden | The service information provided in the request is rejected because the requested MBS service is not authorized. |

### 6.2.8	Feature negotiation

The optional features in table 6.2.8-1 are defined for the Npcf_MBSPolicyAuthorization API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 6.2.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 6.2.9	Security

As indicated in 3GPP TS 33.501 [8] and 3GPP TS 29.500 [4], the access to the Npcf_MBSPolicyAuthorization API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [9]), based on local configuration, using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [10]) plays the role of the authorization server.

If OAuth2 is used, an NF Service Consumer, prior to consuming services offered by the Npcf_MBSPolicyAuthorization API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [10], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Npcf_MBSPolicyAuthorization service.

The Npcf_MBSPolicyAuthorization API defines a single scope "mbspolicyauth" for the entire service, and it does not define any additional scopes at resource or operation level.

Annex A (normative):
OpenAPI specification

# A.1	General

This Annex specifies the formal definition of the API(s) defined in the present specification. It consists of OpenAPI  specifications in YAML format.

This Annex takes precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API(s).

NOTE 1:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification files contained in this 3GPP Technical Specification are available on a Git-based repository that uses the GitLab software version control system (see 3GPP TS 29.501 [5] clause 5.3.1 and 3GPP TR 21.900 [7] clause 5B).

# A.2	Npcf_MBSPolicyControl API

openapi: 3.0.0

info:

title: Npcf_MBSPolicyControl API

version: 1.0.0-alpha.3

description: |

MBS Session Policy Control Service

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.537 V0.3.0; 5G System; Multicast/Broadcast Policy Control Services.

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.537/'

security:

- {}

- oAuth2ClientCredentials:

- npcf-mbspolicycontrol

servers:

- url: '{apiRoot}/npcf-mbspolicycontrol/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501.

paths:

/mbs-policies:

post:

summary: Request the creation of a new Individual MBS Policy resource.

operationId: CreateMBSPolicy

tags:

- MBS Policies (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MbsPolicyCtxtData'

responses:

'201':

description: >

Created. An Individual MBS Policy resource is successfully created and

a representation of the created resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/MbsPolicyDecision'

headers:

Location:

description: >

Contains the URI of the newly created Individual MBS Policy resource.

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

MbsPolicyUpdateNotification:

'{$request.body#/notificationUri}/update':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MbsPolicyNotif'

responses:

'204':

description: >

No Content. The MBS Policy Update Notification was successfully received.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

MbsPolicyTerminationNotification:

'{$request.body#/notificationUri}/terminate':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MbsTermNotif'

responses:

'204':

description: >

No Content. The notification was successfully received.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/mbs-policies/{mbsPolicyId}:

get:

summary: Read an Individual MBS Policy.

operationId: GetMBSPolicy

tags:

- Individual MBS Policy (Document)

parameters:

- name: mbsPolicyId

in: path

description: >

Contains the identifier of the concerned Individual MBS Session Policy Association.

required: true

schema:

type: string

responses:

'200':

description: >

OK. The requested Individual MBS Policy resource is successfully returned.

content:

application/json:

schema:

$ref: '#/components/schemas/MbsPolicyData'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Deletes an existing Individual MBS Policy resource.

operationId: DeleteMBSPolicy

tags:

- Individual MBS Policy

parameters:

- name: mbsPolicyId

in: path

description: >

Contains the identifier of the concerned Individual MBS Session Policy.

required: true

schema:

type: string

responses:

'204':

description: >

No Content. The concerned Individual MBS Policy resource was successfully deleted.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

npcf-mbspolicycontrol: Access to the Npcf_MBSPolicyControl API

schemas:

MbsPolicyCtxtData:

description: >

Contains the parameters used to request the creation of an Individual MBS

Policy resource.

type: object

properties:

mbsSessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsSessionId'

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

notificationUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- mbsSessionId

- dnn

- snssai

- notificationUri

MbsPolicyData:

description: Contains the MBS policy data of an Individual MBS Policy resource.

type: object

properties:

mbsPolicyCtxtData:

$ref: '#/components/schemas/MbsPolicyCtxtData'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- mbsPolicyCtxtData

MbsPolicyDecision:

description: >

Identifies the service requirements of an Individual Application MBS Session Context.

type: object

properties:

mbsPccRules:

type: object

additionalProperties:

$ref: '#/components/schemas/MbsPccRule'

minProperties: 1

description: >

A map of MBS PCC rules with the content being the MbsPccRule

The key used in this map for each entry is the pccRuleId attribute of the corresponding

MbsPccRule.

mbsQosInfos:

$ref: '#/components/schemas/MbsQosInfo'

mbsQosChars:

type: object

additionalProperties:

$ref: '#/components/schemas/MbsQosChar'

minProperties: 1

description: >

Map of QoS characteristics for non standard 5QIs. This map uses the 5QI values as keys.

MbsPolicyNotif:

description: Represents an MBS Policy Update Notification.

type: object

properties:

mbsSessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsSessionId'

required:

- mbsSessionId

MbsTermNotif:

description: Represents an MBS Policy Termination Notification.

type: object

properties:

mbsSessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsSessionId'

cause:

$ref: '#/components/schemas/MbsPolicyAssociationReleaseCause'

required:

- mbsSessionId

- cause

MbsPccRule:

description: Contains a PCC rule information for MBS sessions.

type: object

properties:

dlIpFlowInfos:

type: array

items:

$ref: 'TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/FlowDescription'

minItems: 1

description: An array of downlink IP flow packet filter information.

pccRuleId:

type: string

description: Univocally identifies the PCC rule within a MBS session.

precedence:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

refMbsQosInfo:

type: array

items:

type: string

minItems: 1

maxItems: 1

description: >

A reference to the MbsQosInfo policy decision type.

MbsQosInfo:

description: Contains MBS session QoS requirements.

type: object

properties:

mbsQosId:

type: string

description: Univocally identifies the QoS control policy data within a MBS session.

5qi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/5Qi'

mbrDl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

gbrDl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

arp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Arp'

priorityLevel:

$ref: 'TS29571_CommonData.yaml#/components/schemas/5QiPriorityLevel'

averWindow:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AverWindow'

required:

- mbsQosId

MbsQosChar:

description: Represents explicitly signalled  QoS characteristics.

type: object

properties:

5qi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/5Qi'

resourceType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/QosResourceType'

priorityLevel:

$ref: 'TS29571_CommonData.yaml#/components/schemas/5QiPriorityLevel'

packetDelayBudget:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketDelBudget'

packetErrorRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketErrRate'

averWindow:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AverWindow'

required:

- 5qi

- resourceType

- priorityLevel

- packetDelayBudget

- packetErrorRate

# ENUMS:

MbsPolicyAssociationReleaseCause:

description: >

Represents the cause due to which the PCF requests the termination of the MBS policy

association.

anyOf:

- type: string

enum:

- UNSPECIFIED

# A.3	Npcf_MBSPolicyAuthorization API

openapi: 3.0.0

info:

title: Npcf_MBSPolicyAuthorization API

version: 1.0.0-alpha.1

description: |

MBS Policy Authorization Service.

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.537 V0.2.0; 5G System; Multicast/Broadcast Policy Control Services.

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.537/'

security:

- {}

- oAuth2ClientCredentials:

- npcf-mbspolicyauth

servers:

- url: '{apiRoot}/npcf-mbspolicyauth/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in subclause 4.4 of 3GPP TS 29.501.

paths:

/mbs-app-sessions:

post:

summary: Request the creation of a new Individual MBS Application Session Context resource.

operationId: CreateMBSAppSessionCtxt

tags:

- MBS Application Session Contexts (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MbsAppSessionCtxt'

responses:

'201':

description: >

Created. An Individual MBS Application Session Context resource is successfully

created and a representation of the created resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/MbsAppSessionCtxt'

headers:

Location:

description: >

Contains the URI of the newly created Individual MBS Application Session

Context resource.

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

description: Not Found

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/mbs-app-sessions/{mbsAppSessionId}:

get:

summary: Read an existing Individual MBS Application Session Context resource.

operationId: GetMBSAppSessionCtxt

tags:

- Individual MBS Application Session Context (Document)

parameters:

- name: mbsAppSessionId

in: path

description: >

Contains the identifier of the targeted Individual MBS Application Session

Context.

required: true

schema:

type: string

responses:

'200':

description: >

OK. The requested Individual MBS Application Session Context resource is

successfully returned.

content:

application/json:

schema:

$ref: '#/components/schemas/MbsAppSessionCtxt'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: Modify an existing Individual MBS Application Session Context resource.

operationId: ModifyMBSAppSessionCtxt

tags:

- Individual MBS Application Session Context (Document)

parameters:

- name: mbsAppSessionId

in: path

description: >

Contains the identifier of the targeted Individual MBS Application Session

Context.

required: true

schema:

type: string

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/MbsAppSessionCtxtPatch'

responses:

'200':

description: >

OK. The requested Individual MBS Application Session Context resource is

successfully modified and a representation of the updated resource is

returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/MbsAppSessionCtxt'

'204':

description: >

No Content. The concerned Individual MBS Application Session Context resource is

successfully modified and no content is returned in the response body.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Delete an existing Individual MBS Application Session Context resource.

operationId: DeleteMBSAppSessionCtxt

tags:

- Individual MBS Application Session Context (Document)

parameters:

- name: mbsAppSessionId

in: path

description: >

Contains the identifier of the targeted Individual MBS Application Session

Context.

required: true

schema:

type: string

responses:

'204':

description: >

No Content. The concerned Individual MBS Application Session Context resource is

successfully deleted.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

npcf-mbspolicyauth: Access to the Npcf_MBSPolicyAuthorization API

schemas:

MbsAppSessionCtxt:

description: >

Identifies the service requirements of an Individual MBS Application Session Context.

type: object

properties:

mbsSessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsSessionId'

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- mbsSessionId

MbsAppSessionCtxtPatch:

description: >

Describes the modifications to an existing Individual MBS Application Session

Context resource.

type: string

# Editor’s Note: The definition of the MbsAppSessionCtxtPatch data type is FFS.

Annex B (informative):
Withdrawn API versions

# B.1	General

This Annex lists withdrawn API versions of the APIs defined in the present specification. Clause 4.3.1.6 of 3GPP TS 29.501 [5] describes the withdrawal of API versions.

# B.2	Npcf_MBSPolicyControl API

The API versions listed in table B.2-1 are withdrawn for the Npcf_MBSPolicyControl API.

Table B.2-1: Withdrawn API versions of the Npcf_MBSPolicyControl service

| API version number | Remarks |
| --- | --- |

# B.3	Npcf_MBSPolicyAuthorization API

The API versions listed in table B.3-1 are withdrawn for the Npcf_MBSPolicyAuthorization API.

Table B.3-1: Withdrawn API versions of the Npcf_MBSPolicyAuthorization service

| API version number | Remarks |
| --- | --- |

Annex C (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-01 | CT3#119bis-e | C3-220401 |  |  |  | TS Skeleton | 0.0.0 |
| 2022-01 | CT3#119bis-e | C3-220457 |  |  |  | Inclusion of C3-220113, C3-220114, C3-220115, C3-220116, C3-220402, C3-220403, C3-220404 C3-220491, C3-220492, C3-220493 | 0.1.0 |
| 2022-04 | CT3#121-e | C3-222483 |  |  |  | Inclusion of C3-222056, C3-222124, C3-222127, C3-222403, C3-222404, C3-222405, C3-222406 C3-222407, C3-222462 | 0.2.0 |
| 2022-05 | CT3#122-e | C3-223506 |  |  |  | Inclusion of C3-223561, C3-223574, C3-223562, C3-223563, C3-223564, C3-223568, C3-223565 C3-223566, C3-223573, C3-223308 | 0.3.0 |