| 3GPP TS 29.555 V0.2.0 (2021-06) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; 5G System; 5G Direct Discovery Name Management Services; Stage 3 (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

1	Scope	7

2	References	7

3	Definitions, symbols and abbreviations	8

3.1	Definitions	8

3.2	Symbols	8

3.3	Abbreviations	8

4	Overview	8

5	Services offered by the 5G DDNMF	9

5.1	Introduction	9

5.2	N5g-ddnmf_Discovery Service	10

5.2.1	Service Description	10

5.2.2	Service Operations	10

5.2.2.1	Introduction	10

5.2.2.2	AnnounceAuthorize	10

5.2.2.2.1	General	10

5.2.2.2.2	Obtain the authorization to announce for a UE	10

5.2.2.3	AnnounceUpdate	11

5.2.2.3.1	General	11

5.2.2.3.2	Update the authorization for announcing for a UE	11

5.2.2.4	MonitorAuthorize	12

5.2.2.4.1	General	12

5.2.2.4.2	Obtain the authorization to monitor for a UE	12

5.2.2.5	MonitorUpdate	12

5.2.2.5.1	General	12

5.2.2.5.2	Update the authorization for monitoring for a UE	13

5.2.2.6	MonitorUpdateResult	13

5.2.2.6.1	General	13

5.2.2.6.2	Monitor Update Result Notification	13

5.2.2.7	DiscoveryAuthorize	14

5.2.2.7.1	General	14

5.2.2.7.2	Obtain the authorization for a discoverer UE to operate Model B restricted discovery	14

5.2.2.8	MatchReport	14

5.2.2.8.1	General	14

5.2.2.8.2	Obtain the information about the indicated discovery code	14

5.2.2.9	MatchInformation	15

5.2.2.9.1	General	15

5.2.2.9.2	Match Information Notification	15

6	API Definitions	16

6.1	N5g-ddnmf_Discovery Service API	16

6.1.1	Introduction	16

6.1.2	Usage of HTTP	16

6.1.2.1	General	16

6.1.2.2	HTTP standard headers	16

6.1.2.2.1	General	16

6.1.2.2.2	Content type	16

6.1.2.3	HTTP custom headers	17

6.1.3	Resources	17

6.1.3.1	Overview	17

6.1.3.2	Resource: AnnounceData	18

6.1.3.2.1	Description	18

6.1.3.2.2	Resource Definition	18

6.1.3.2.3	Resource Standard Methods	18

6.1.3.2.3.1	PUT	18

6.1.3.2.3.2	PATCH	19

6.1.3.3	Resource: MonitorData	20

6.1.3.3.1	Description	20

6.1.3.3.2	Resource Definition	20

6.1.3.3.3	Resource Standard Methods	20

6.1.3.3.3.1	PUT	20

6.1.3.3.3.2	PATCH	21

6.1.3.4	Resource:  DiscoveryData	21

6.1.3.4.1	Description	21

6.1.3.4.2	Resource Definition	21

6.1.3.4.3	Resource Standard Methods	21

6.1.3.4.3.1	PUT	21

6.1.3.5	Resource: UeData (Custom operation)	22

6.1.3.5.1	Description	22

6.1.3.5.2	Resource Definition	22

6.1.3.5.3	Resource Standard Methods	22

6.1.3.5.4	Resource Custom Operations	22

6.1.3.5.4.1	Overview	22

6.1.3.5.4.2	Operation: match-report	23

6.1.3.5.4.2.1	Description	23

6.1.3.5.4.2.2	Operation Definition	23

6.1.4	Custom Operations without associated resources	23

6.1.5	Notifications	23

6.1.5.1	General	23

6.1.5.2	MonitorUpdateResult	23

6.1.5.2.1	Description	23

6.1.5.2.2	Target URI	24

6.1.5.2.3	Standard Methods	24

6.1.5.2.3.1	POST	24

6.1.5.3	MatchInformation	24

6.1.5.3.1	Description	24

6.1.5.3.2	Target URI	24

6.1.5.3.3	Standard Methods	24

6.1.5.3.3.1	POST	24

6.1.6	Data Model	25

6.1.6.1	General	25

6.1.6.2	Structured data types	27

6.1.6.2.1	Introduction	27

6.1.6.2.2	Type: AnnounceAuthReqData	27

6.1.6.2.3	Type: AnnounceAuthRespData	27

6.1.6.2.4	Type: AnnounceDiscDataForOpen	28

6.1.6.2.5	Type: AnnounceDiscDataForRestricted	28

6.1.6.2.6	Type: AnnounceUpdateData	29

6.1.6.2.7	Type: MonitorAuthReqData	29

6.1.6.2.8	Type: MonitorAuthRespData	30

6.1.6.2.9	Type: MonitorDiscDataForOpen	30

6.1.6.2.10	Type: MonitorDiscDataForRestricted	30

6.1.6.2.11	Type: MonitorAuthDataForOpen	31

6.1.6.2.12	Type: MonitorAuthDataForRestricted	31

6.1.6.2.13	Type: MonitorUpdateData	31

6.1.6.2.14	Type: DiscoveryAuthReqData	32

6.1.6.2.15	Type: DiscoveryAuthRespData	32

6.1.6.2.16	Type: DiscDataForRestricted	32

6.1.6.2.17	Type: AuthDataForRestricted	33

6.1.6.2.18	Type: MatchReportReqData	33

6.1.6.2.19	Type: MatchReportRespData	33

6.1.6.2.20	Type: MonitorUpdateResult	34

6.1.6.2.21	Type: MatchInformation	34

6.1.6.2.22	Type: MatchInfoForOpen	34

6.1.6.2.23	Type: MatchInfoForRestricted	35

6.1.6.3	Simple data types and enumerations	35

6.1.6.3.1	Introduction	35

6.1.6.3.2	Simple data types	35

6.1.6.3.3	Enumeration: DiscoveryType	35

6.1.6.3.4	Enumeration: AuthorizationResult	36

6.1.6.3.5	Enumeration: RevocationResult	36

6.1.6.4	Data types describing alternative data types or combinations of data types	36

6.1.6.5	Binary data	36

6.1.7	Error Handling	36

6.1.7.1	General	36

6.1.7.2	Protocol Errors	36

6.1.7.3	Application Errors	36

6.1.8	Feature negotiation	37

6.1.9	Security	37

Annex A (normative): OpenAPI specification	38

A.1	General	38

A.2	N5g-ddnmf_Discovery API	38

Annex B (informative): Change history	49

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the stage 3 protocol and data model for the N5g-ddnmf Service Based Interface. It provides stage 3 protocol definitions and message flows, and specifies the API for each service offered by the 5G DDNMF as specified in 3GPP TS 23.304 [4].

The 5G System stage 2 architecture and procedures are specified in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3].

The Technical Realization of the Service Based Architecture and the Principles and Guidelines for Services Definition are specified in 3GPP TS 29.500 [5] and 3GPP TS 29.501 [6].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[5]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[6]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[7]	OpenAPI : "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[8]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[9]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[10]	IETF RFC 7807: "Problem Details for HTTP APIs".

[11]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[12]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[13]	3GPP TS 29.510: "Network Function Repository Services; Stage 3".

[14]	3GPP TR 21.900: "Technical Specification Group working methods".

[15]	IETF RFC 7396: "JSON Merge Patch".

[16]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces Stage 3".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

5G DDNMF: The 5G DDNMF is the logical function handling network related actions required for dynamic ProSe Direct Discovery. The 5G DDNMF in the HPLMN may interact with the 5G DDNMF in a VPLMN or Local PLMN in order to manage the ProSe Direct Discovery service.

ProSe Direct Discovery: A procedure employed by a ProSe-enabled UE to discover other ProSe-enabled UEs in its vicinity based on direct radio transmissions between the two UEs with NR technology.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5G DDNMF	5G Direct Discovery Name Management Function

5G ProSe	5G Proximity-based Services

RPAUID	Restricted ProSe Application User ID

PDUID	ProSe Discovery UE ID

# 4	Overview

The 5G Direct Discovery Name Management Function (5G DDNMF) is the network entity in the 5G Core Network (5GC) supporting Direct Discovery Services. Within the 5GC, the 5G DDNMF in HPLMN invokes services provided by the 5G DDNMF in Local PLMN and/or in VPLMN via the N5g-ddnmf service based interface (see 3GPP TS 23.304 [4]).

Figure 4-1 provides the reference model (in service based interface representation and in reference point representation), with focus on the 5G DDNMF:

Figure 4-1: Reference model – 5G DDNMF

The functionalities supported by the 5G DDNMF are listed in clause 4.3.2 of 3GPP TS 23.304 [4].

NOTE: Only service based interfaces between 5G DDNMFs will be covered in this TS, other interfaces won't be covered for the time being.

# 5	Services offered by the 5G DDNMF

## 5.1	Introduction

The table 5.1-1 shows the 5G DDNMF Services and 5G DDNMF Service Operations:

Table 5.1-1: List of 5G DDNMF Services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| N5g-ddnmf_Discovery | AnnounceAuthorize | Request/Response | 5G DDNMF |
|  | AnnounceUpdate | Request/Response | 5G DDNMF |
|  | MonitorAuthorize | Request/Response | 5G DDNMF |
|  | MonitorUpdate | Request/Response | 5G DDNMF |
|  | MonitorUpdateResult | Notify | 5G DDNMF |
|  | DiscoveryAuthorize | Request/Response | 5G DDNMF |
|  | MatchReport | Request/Response | 5G DDNMF |
|  | MatchInformation | Notify | 5G DDNMF |

Table 5.1-2 summarizes the corresponding APIs defined for this specification.

Table 5.1-2: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |
| N5g-ddnmf_Discovery | 6.1 | N5g-ddnmf Discovery Service | TS29555_ N5g-ddnmf_Discovery.yaml | n5gddnmf-disc | A.2 |

## 5.2	N5g-ddnmf_Discovery Service

### 5.2.1	Service Description

The N5g-ddnmf_Discovery service enables an NF or SCP to manage inter-PLMN ProSe Direct Discovery operations for a target UE. The following are the key functionalities of this NF service.

-	Allow the consumer NF to obtain the authorization from the 5G DDNMF for announcing in the PLMN.

-	Allow the consumer NF to update or revoke the authorization from the 5G DDNMF for announcing in the PLMN.

-	Allow the consumer NF to obtain the authorization from the 5G DDNMF for monitoring in the PLMN.

-	Allow the consumer NF to update or revoke the authorization for the indicated UE to monitor in the PLMN.

-	Allow the consumer NF to inform the 5G DDNMF of the monitoring revocation results.

-	Allow the consumer NF to obtain the authorization from the 5G DDNMF for a discoverer UE in the PLMN to operate Model B restricted discovery.

-	Allow the consumer NF to obtain the information about the indicated discovery code from the 5G DDNMF.

-	Allow the consumer NF to receive from the 5G DDNMF of a matching result, and the information can be used for charging purpose.

### 5.2.2	Service Operations

#### 5.2.2.1	Introduction

This clause introduces the service operations defined for the N5g-ddnmf_Discovery services as follows:

-	AnnounceAuthorize

-	AnnounceUpdate

-	MonitorAuthorize

-	MonitorUpdate

-	MonitorUpdateResult

-	DiscoveryAuthorize

-	MatchReport

-	MatchInformation

#### 5.2.2.2	AnnounceAuthorize

##### 5.2.2.2.1	General

The AnnounceAuthorize service operation shall be used by the NF Service consumer to obtain the authorization to announce for a UE from the 5G DDNMF in the PLMN. The following procedures are supported using the AnnounceAuthorize service operation:

-	Discovery Request procedures (see 3GPP 3GPP TS 23.304 [4], clause 6.3.1.4)

##### 5.2.2.2.2	Obtain the authorization to announce for a UE

The AnnounceAuthorize service operation is invoked by a NF Service Consumer, e.g. HPLMN 5G DDNMF, towards the 5G DDNMF (e.g. VPLMN 5G DDNMF or Local PLMN 5G DDNMF) to request to obtain the authorization from the 5G DDNMF for announcing for a target UE. See Figure 5.2.2.2.2-1. The request contains the UE's identity (/{ueId}) which shall be a SUPI or GPSI and the discovery Entry ID (/{discEntryId}) which is used to identify the discovery entry related to this request.

Figure 5.2.2.2.2-1: Obtain the authorization to announce for a UE

1.	The NF Service Consumer shall send an HTTP PUT request to the resource representing the authorization to announce for a UE to obtain the authorization to announce for this UE. The request shall include the Discovery Type, if the Discovery Type is OPEN the Announce Authorisation Data for open discovery shall be included, and if the Discovery Type is RESTRICTED the Announce Authorisation Data for restricted discovery shall be included in the HTTP PUT request body.

2a.	On success, "201 created" shall be returned. The response body shall contain the parameters related to the determined authorization data to announce for the UE.

2b	On failure, one of the HTTP status code listed in Table 6.1.3.2.3.1-3 may be returned. For a 4xx/5xx response, the message body may contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.3.2.3.1-3.

#### 5.2.2.3	AnnounceUpdate

##### 5.2.2.3.1	General

The AnnounceUpdate service operation shall be used by the NF Service consumer to update or revoke the authorization from the 5G DDNMF for announcing in the PLMN. The following procedures are supported using the AnnounceUpdate service operation:

-	Discovery Request procedures (see 3GPP 3GPP TS 23.304 [4], clause 6.3.1.4)

##### 5.2.2.3.2	Update the authorization for announcing for a UE

The AnnounceUpdate service operation is invoked by a NF Service Consumer, e.g. HPLMN 5G DDNMF, towards the 5G DDNMF (e.g. VPLMN 5G DDNMF or Local PLMN 5G DDNMF) to request to update the authorization for announcing in the PLMN from the 5G DDNMF for a target UE. See Figure 5.2.2.3.2-1. The request contains the UE's identity (/{ueId}) which shall be a SUPI or GPSI and the discovery Entry ID (/{discEntryId}) which is used to identify the discovery entry related to this request.

Figure 5.2.2.3.2-1: Update the authorization for announcing for a UE

1.	The NF Service Consumer shall send an HTTP PATCH request to the resource representing the authorization to announce for a UE to update or revoke the authorization from the 5G DDNMF for announcing in the PLMN. The request shall include Discovery Type, the Validity Timer, and the ProSe Application Code if the ProSe Application Code is changed in the HTTP PATCH request body. If the Validity Timer sets to a full zero, it indicates to revoke the authorization for the announcing in the PLMN.

2a.	On success, "204 No Content" shall be returned.

#### 5.2.2.4	MonitorAuthorize

##### 5.2.2.4.1	General

The MonitorAuthorize service operation shall be used by the NF Service consumer to obtain the authorization from the 5G DDNMF for monitoring for an UE in the PLMN. The following procedures are supported using the MonitorAuthorize service operation:

-	Discovery Request procedures (see 3GPP 3GPP TS 23.304 [4], clause 6.3.1.4).

##### 5.2.2.4.2	Obtain the authorization to monitor for a UE

The MonitorAuthorize service operation is invoked by a NF Service Consumer, e.g. HPLMN 5G DDNMF, towards the 5G DDNMF (e.g. VPLMN 5G DDNMF or Local PLMN 5G DDNMF) to request to obtain the authorization from the 5G DDNMF for monitoring for a target UE. See Figure 5.2.2.4.2-1. The request contains the UE's identity (/{ueId}) which shall be a SUPI or GPSI and the discovery Entry ID (/{discEntryId}) which is used to identify the discovery entry related to this request.

Figure 5.2.2.4.2-1: Obtain the authorization to monitor for a UE

1.	The NF Service Consumer shall send an HTTP PUT request to the resource representing the authorization to monitor for a UE to obtain the authorization to monitor for this UE. The request shall include the Discovery Type, if the Discovery Type is OPEN the Monitor Authorisation Data for open discovery shall be included, and if the Discovery Type is RESTRICTED the Monitor Authorisation Data for restricted discovery shall be included in the HTTP PUT request body.

2a.	On success, "201 created" shall be returned. The response body shall contain the parameters related to the determined authorization data to monitor for the UE.

2b	On failure, one of the HTTP status code listed in Table 6.1.3.3.3.1-3 may be returned. For a 4xx/5xx response, the message body may contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.3.3.3.1-3.

#### 5.2.2.5	MonitorUpdate

##### 5.2.2.5.1	General

The MonitorUpdate service operation shall be used by the NF Service consumer to update or revoke the authorization for the indicated UE to monitor in the PLMN. The following procedures are supported using the MonitorUpdate service operation:

-	Discovery Request procedures (see 3GPP 3GPP TS 23.304 [4], clause 6.3.1.4).

##### 5.2.2.5.2	Update the authorization for monitoring for a UE

The MonitorUpdate service operation is invoked by a NF Service Consumer, e.g. HPLMN 5G DDNMF, towards the 5G DDNMF (e.g. VPLMN 5G DDNMF or Local PLMN 5G DDNMF) to request to update the authorization for monitoring in the PLMN from the 5G DDNMF for a target UE. See Figure 5.2.2.5.2-1. The request contains the UE's identity (/{ueId}) which shall be a SUPI or GPSI and the discovery Entry ID (/{discEntryId}) which is used to identify the discovery entry related to this request.

Figure 5.2.2.5.2-1: Update the authorization for monitoring for a UE

1.	The NF Service Consumer shall send an HTTP PATCH request to the resource representing the authorization to monitor for a UE to update or revoke the authorization for the indicated UE to monitor in the PLMN. The request shall include Discovery Type, the ProSe Application ID Name, and the TTL if the ProSe Application Code is changed in the HTTP PATCH request body. If the value of TTL sets to zero, it indicates to revoke the authorization for the monitoring in the PLMN, and monitorUpdateResultCallbackRef may be included in the request body if the NF Service Consumer expects to receive the monitoring revocation results

2a.	On success, "204 No Content" shall be returned.

#### 5.2.2.6	MonitorUpdateResult

##### 5.2.2.6.1	General

The MonitorUpdateResult service operation shall be used by the 5G DDNMF to notify the NF Service consumer of a the 5G DDNMF of the monitoring revocation results. The following procedures are supported using the MonitorUpdateResult service operation:

-	Direct Discovery Update Procedures (see 3GPP 3GPP TS 23.304 [4], clause 6.3.1.7).

##### 5.2.2.6.2	Monitor Update Result Notification

The MonitorUpdateResult service operation notifies the NF service consumer (e.g. HPLMN 5G DDNMF) serving the user about the monitoring revocation results for the user. The request contains the monitorUpdateResultCallbackRef URI. See Figure 5.2.2.6.2-1.

Figure 5.2.2.6.2-1: Monitor Update Result Notification

1.	The 5G DDNMF sends a POST request to the monitorUpdateResultCallbackRef to notify the NF service consumer about the monitoring revocation results for the user. The request shall the Discovery Type, the ProSe Restricted Code, the Application ID, the Banned RPAUID, the Banned PDUID, and the monitoring revocation results.

2.	The NF service consumer responds with "204 No Content".

#### 5.2.2.7	DiscoveryAuthorize

##### 5.2.2.7.1	General

The DiscoveryAuthorize service operation shall be used by the NF Service consumer to obtain the authorization from the 5G DDNMF for a discoverer UE in the PLMN to operate Model B restricted discovery. The following procedures are supported using the DiscoveryAuthorize service operation:

-	Discovery Request procedures (see 3GPP 3GPP TS 23.304 [4], clause 6.3.1.4).

##### 5.2.2.7.2	Obtain the authorization for a discoverer UE to operate Model B restricted discovery

The DiscoveryAuthorize service operation is invoked by a NF Service Consumer, e.g. HPLMN 5G DDNMF, towards the 5G DDNMF (e.g. VPLMN 5G DDNMF or Local PLMN 5G DDNMF) to request to obtain the authorization from the 5G DDNMF for a discoverer UE in the PLMN to operate Model B restricted discovery. See Figure 5.2.2.7.2-1. The request contains the UE's identity (/{ueId}) which shall be a SUPI or GPSI and the discovery Entry ID (/{discEntryId}) which is used to identify the discovery entry related to this request.

Figure 5.2.2.7.2-1: Obtain the authorization for a discoverer UE to operate Model B restricted discovery

1.	The NF Service Consumer shall send an HTTP PUT request to the resource representing the authorization for a discoverer UE to obtain the authorization for a discoverer UE to operate Model B restricted discovery. The request shall include the Discovery Type, authorisation data for restricted discovery in the HTTP PUT request body.

2a.	On success, "201 created" shall be returned. The response body shall contain the parameters related to the determined authorization data for the discoverer UE to operate Model B restricted discovery.

2b	On failure, one of the HTTP status code listed in Table 6.1.3.4.3.1-3 may be returned. For a 4xx/5xx response, the message body may contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.3.4.3.1-3.

#### 5.2.2.8	MatchReport

##### 5.2.2.8.1	General

The MatchReport service operation shall be used by the NF Service consumer to obtain the information about the indicated discovery code from the 5G DDNMF. The following procedures are supported using the MatchReport service operation:

-	Discovery Reporting procedures (see 3GPP 3GPP TS 23.304 [4], clause 6.3.1.5).

##### 5.2.2.8.2	Obtain the information about the indicated discovery code

The MatchReport service operation is invoked by a NF Service Consumer, e.g. HPLMN 5G DDNMF, towards the 5G DDNMF (e.g. VPLMN 5G DDNMF or Local PLMN 5G DDNMF) to request to obtain the information about the indicated discovery code from the 5G DDNMF. See Figure 5.2.2.8.2-1. The request contains the UE's identity (/{ueId}) which shall be a SUPI or GPSI, the type of request (/match-report).

Figure 5.2.2.8.2-1: Obtain the information about the indicated discovery code

1.	The NF Service Consumer shall send an HTTP POST request to the resource representing the information about the indicated discovery code to obtain the information about the indicated discovery code. The request shall include the Discovery Type, the ProSe Application Codes if the discovery type is OPEN in the HTTP POST request body, and optionally includes Monitored PLMN ID in the HTTP POST request body.

2a.	On success, "200 OK" shall be returned. The response body shall contain the parameters related to the information about the indicated discovery code.

2b	On failure, one of the HTTP status code listed in Table 6.1.3.5.3.1-3 may be returned. For a 4xx/5xx response, the message body may contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.3.5.3.1-3.

#### 5.2.2.9	MatchInformation

##### 5.2.2.9.1	General

The MatchInformation service operation shall be used by the 5G DDNMF to notify the NF Service consumer of a matching result, and the information that can be used for charging purpose. The following procedures are supported using the MatchInformation service operation:

-	Discovery Reporting procedures (see 3GPP 3GPP TS 23.304 [4], clause 6.3.1.5)

##### 5.2.2.9.2	Match Information Notification

The MatchInformation service operation notifies the NF service consumer (e.g. VPLMN 5G DDNMF or Local PLMN 5G DDNMF) about match information including a matching result, and the information can be used for charging purpose. The request contains the matchInfoCallbackRef URI. See Figure 5.2.2.9.2-1.

Figure 5.2.2.9.2-1: Match Information Notification

1.	The 5G DDNMF sends a POST request to the matchInfoCallbackRef URI to notify the NF service consumer about match information including a matching result, and the information can be used for charging purpose. The request shall include the Discovery Type, match report information for open discovery type if match report information for open discovery type is OPEN, and match report information for restricted discovery type if match report information for open discovery type is RESTRICTED

2.	The NF service consumer responds with "204 No Content".

# 6	API Definitions

## 6.1	N5g-ddnmf_Discovery Service API

### 6.1.1	Introduction

The N5g-ddnmf_Discovery shall use the N5g-ddnmf_Discovery API.

The API URI of the N5g-ddnmf_Discovery API shall be:

{apiRoot}/<apiName>/<apiVersion>/

The request URIs used in HTTP requests from the NF service consumer towards the NF service producer shall have the Resource URI structure defined in clause 4.4.1 of 3GPP TS 29.501 [6], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [6].

-	The <apiName> shall be "n5g-ddnmf-disc".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 6.1.3.

### 6.1.2	Usage of HTTP

#### 6.1.2.1	General

HTTP/2, IETF RFC 7540 [8], shall be used as specified in clause 5 of 3GPP TS 29.500 [5].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [5].

The OpenAPI [7] specification of HTTP messages and content bodies for the n5g-ddnmf-disc API is contained in Annex A.2.

#### 6.1.2.2	HTTP standard headers

##### 6.1.2.2.1	General

See clause 5.2.2 of 3GPP TS 29.500 [5] for the usage of HTTP standard headers.

##### 6.1.2.2.2	Content type

JSON, IETF RFC 8259 [9], shall be used as content type of the HTTP bodies specified in the present specification as specified in clause 5.4 of 3GPP TS 29.500 [5]. The use of the JSON format shall be signalled by the content type "application/json".

"Problem Details" JSON object shall be used to indicate additional details of the error in a HTTP response body and shall be signalled by the content type "application/problem+json", as defined in IETF RFC 7807 [10].

JSON Merge Patch, as defined in IETF RFC 7396 [15], signalled by the content type "application/merge-patch+json".

#### 6.1.2.3	HTTP custom headers

The mandatory HTTP custom header fields specified in clause 5.2.3.2 of 3GPP TS 29.500 [5] shall be supported, and the optional HTTP custom header fields specified in clause 5.2.3.3 of 3GPP TS 29.500 [4] may be supported.

### 6.1.3	Resources

#### 6.1.3.1	Overview

Figure 6.1.3.1-1: Resource URI structure of the N5g-ddnmf_Discovery API

Table 6.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.1.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| AnnounceData | /{ueId}/announce-authorize/{discEntryId} | PUT | Obtain the authorization to announce for a UE from the 5G DDNMF in the PLMN. |
|  |  | PATCH | Update or revoke the authorization from the 5G DDNMF for announcing in the PLMN. |
| MonitorData | /{ueId}/monitor-authorize/{discEntryId} | PUT | Obtain the authorization from the 5G DDNMF for monitoring for an UE in the PLMN. |
|  |  | PATCH | Update or revoke the authorization for the indicated UE to monitor in the PLMN. |
| DiscoveryData | /{ueId}/discovery-authorize/{discEntryId} | PUT | Obtain the authorization from the 5G DDNMF for a discoverer UE in the PLMN to operate Model B restricted discovery. |
| UeData | /{ueId}/match-report | match-report (POST) | Obtain the information about the indicated discovery code from the 5G DDNMF. |

#### 6.1.3.2	Resource: AnnounceData

This resource represents the announce data.

##### 6.1.3.2.1	Description

#### 6.1.3.2.2	Resource Definition

Resource URI: {apiRoot}/n5g-ddnmf-disc/<apiVersion>/{ueId}/announce-authorize/{discEntryId}

This resource shall support the resource URI variables defined in table 6.1.3.2.2-1.

Table 6.1.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1 |
| apiVersion | string | See clause 6.1.1 |
| ueId | VarUeId | Represents the Subscription Identifier SUPI or GPSI (see 3GPP TS 23.501 [2] clause 5.9.2) pattern: See pattern of type VarUeId in 3GPP TS 29.571 [16] |
| discEntryId | DiscoveryEntryId | Represents Discovery Entry Id. |

##### 6.1.3.2.3	Resource Standard Methods

###### 6.1.3.2.3.1	PUT

This method shall support the URI query parameters specified in table 6.1.3.2.3.1-1.

Table 6.1.3.2.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.3.2.3.1-3.

Table 6.1.3.2.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AnnounceAuthReqData | M | 1 | Contains the Announce Authorization Data for the indicated UE and indicated discovery entry. |

Table 6.1.3.2.3.1-3: Data structures supported by the <method 1> Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AnnounceAuthRespData | M | 1 | 201 Created | Upon success, a response body containing a representation of the authorized data to announce for the UE shall be returned. |
| NOTE: The manadatory HTTP error status code for the PUT method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [5] also apply. |  |  |  |  |

Table 6.1.3.2.3.1-4: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/n5g-ddnmf-disc>/<apiVersion>/{ueId}/announce-authorize/{discEntryId} |

###### 6.1.3.2.3.2	PATCH

This method shall support the URI query parameters specified in table 6.1.3.2.3.2-1.

Table 6.1.3.2.3.2-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.2.3.2-2 and the response data structures and response codes specified in table 6.1.3.2.3.2-3.

Table 6.1.3.2.3.2-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AnnounceUpdateData | M | 1 | Contains the Announce Authorization Data to update for the indicated UE and indicated discovery entry. |

Table 6.1.3.2.3.2-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| NOTE 1: The manadatory HTTP error status code for the PATCH method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [5] also apply. |  |  |  |  |

#### 6.1.3.3	Resource: MonitorData

##### 6.1.3.3.1	Description

This resource represents the Monitor data.

#### 6.1.3.3.2	Resource Definition

Resource URI: {apiRoot}/n5g-ddnmf-disc/<apiVersion>/{ueId}/monitor-authorize/{discEntryId}

This resource shall support the resource URI variables defined in table 6.1.3.3.2-1.

Table 6.1.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1 |
| apiVersion | string | See clause 6.1.1 |
| ueId | VarUeId | Represents the Subscription Identifier SUPI or GPSI (see 3GPP TS 23.501 [2] clause 5.9.2) pattern: See pattern of type VarUeId in 3GPP TS 29.571 [16] |
| discEntryId | DiscoveryEntryId | Represents Discovery Entry Id. |

##### 6.1.3.3.3	Resource Standard Methods

###### 6.1.3.3.3.1	PUT

This method shall support the URI query parameters specified in table 6.1.3.3.3.1-1.

Table 6.1.3.3.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.1-2 and the response data structures and response codes specified in table 6.1.3.3.3.1-3.

Table 6.1.3.3.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MonitorAuthReqData | M | 1 | Contains the Monitor Authorization Data for the indicated UE and indicated discovery entry. |

Table 6.1.3.3.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MonitorAuthRespData | M | 1 | 201 Created | Upon success, a response body containing a representation of the authorized data to monitor for the UE shall be returned. |
| NOTE: The manadatory HTTP error status code for the PUT method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [5] also apply. |  |  |  |  |

Table 6.1.3.3.3.1-4: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/n5g-ddnmf-disc>/<apiVersion>/{ueId}/monitor-authorize/{discEntryId} |

###### 6.1.3.3.3.2	PATCH

This method shall support the URI query parameters specified in table 6.1.3.3.3.2-1.

Table 6.1.3.3.3.2-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.2-2 and the response data structures and response codes specified in table 6.1.3.3.3.2-3.

Table 6.1.3.3.3.2-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MonitorUpdateData | M | 1 | Contains the Monitor Authorization Data to update for the indicated UE and indicated discovery entry. |

Table 6.1.3.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| NOTE 1: The manadatory HTTP error status code for the PATCH method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [5] also apply. |  |  |  |  |

#### 6.1.3.4	Resource:  DiscoveryData

##### 6.1.3.4.1	Description

This resource represents the Discovery Data.

#### 6.1.3.4.2	Resource Definition

Resource URI: {apiRoot}/n5g-ddnmf-disc/<apiVersion>/{ueId}/discovery-authorize/{discEntryId}

This resource shall support the resource URI variables defined in table 6.1.3.4.2-1.

Table 6.1.3.4.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1 |
| apiVersion | string | See clause 6.1.1 |
| ueId | VarUeId | Represents the Subscription Identifier SUPI or GPSI (see 3GPP TS 23.501 [2] clause 5.9.2) pattern: See pattern of type VarUeId in 3GPP TS 29.571 [16] |
| discEntryId | DiscoveryEntryId | Represents Discovery Entry Id. |

##### 6.1.3.4.3	Resource Standard Methods

###### 6.1.3.4.3.1	PUT

This method shall support the URI query parameters specified in table 6.1.3.4.3.1-1.

Table 6.1.3.4.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.4.3.1-2 and the response data structures and response codes specified in table 6.1.3.4.3.1-3.

Table 6.1.3.4.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DiscoveryAuthReqData | M | 1 | Contains the Discovery Authorization Data for the indicated discoverer UE and indicated discovery entry. |

Table 6.1.3.4.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DiscoveryAuthRespData | M | 1 | 201 Created | Upon success, a response body containing a representation of the authorized data for the discoverer UE in the PLMN to operate Model B restricted discovery shall be returned. |
| NOTE: The manadatory HTTP error status code for the PUT method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [5] also apply. |  |  |  |  |

Table 6.1.3.4.3.1-4: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/n5g-ddnmf-disc>/<apiVersion>/{ueId}/discovery -authorize/{discEntryId} |

#### 6.1.3.5	Resource: UeData (Custom operation)

##### 6.1.3.5.1	Description

This resource represents the UE Data.

#### 6.1.3.5.2	Resource Definition

Resource URI: {apiRoot}/n5g-ddnmf-disc/<apiVersion>/{ueId}

This resource shall support the resource URI variables defined in table 6.1.3.5.2-1.

Table 6.1.3.5.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1 |
| apiVersion | string | See clause 6.1.1 |
| ueId | VarUeId | Represents the Subscription Identifier SUPI or GPSI (see 3GPP TS 23.501 [2] clause 5.9.2) pattern: See pattern of type VarUeId in 3GPP TS 29.571 [16] |

##### 6.1.3.5.3	Resource Standard Methods

##### 6.1.3.5.4	Resource Custom Operations

No Standard Methods are supported for this resource.

###### 6.1.3.5.4.1	Overview

Table 6.1.3.5.4.1-1: Custom operations

| Operation name | Custom operaration URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| match-report | /{ueId}/match-report | POST | Obtain the information about the indicated discovery code from the 5G DDNMF. |

###### 6.1.3.5.4.2	Operation: match-report

6.1.3.5.4.2.1	Description

This operation is used to request the 5G DDNMF to resolve a matched ProSe Discovery Code(s) (ProSe Application Code for open discovery) and obtain the corresponding ProSe Application ID Name(s) and additional information, e.g. metadata.

6.1.3.5.4.2.2	Operation Definition

This operation shall support the request data structures specified in table 6.1.3.5.4.2.2-1 and the response data structure and response codes specified in table 6.1.3.5.4.2.2-2.

Table 6.1.3.5.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MatchReportReqData | M | 1 | Contains the Match Report information. |

Table 6.1.3.5.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MatchReportRespData | M | 1 | 200 OK | Upon success, a response body containing a representation of the Match Report Acknowledgement shall be returned. |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [5] also apply. |  |  |  |  |

### 6.1.4	Custom Operations without associated resources

In this release of this specification, no custom operations without associated resources are defined for the Nudm_SubscriberDataManagement Service.

### 6.1.5	Notifications

#### 6.1.5.1	General

Notifications shall comply to clause 6.2 of 3GPP TS 29.500 [5] and clause 4.6.2.3 of 3GPP TS 29.501 [6].

Table 6.1.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| MonitorUpdateResult | {monitorUpdateResultCallbackRef} | POST | Report the monitoring revocation results. |
| MatchInformation | {matchInfoCallbackRef} | POST | Report an observed matching result to a NF service consumer |

#### 6.1.5.2	MonitorUpdateResult

##### 6.1.5.2.1	Description

The MonitorUpdateResult is used by the 5G DDNMF to report the monitoring revocation results (e.g. 5G DDNMF)

##### 6.1.5.2.2	Target URI

The Callback URI "{monitorUpdateResultCallbackRef}" shall be used with the callback URI variables defined in table 6.1.5.2.2-1.

Table 6.1.5.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| monitorUpdateResultCallbackRef | String formatted as URI with the Callback Uri |

##### 6.1.5.2.3	Standard Methods

###### 6.1.5.2.3.1	POST

This method shall support the request data structures specified in table 6.1.5.2.3.1-1 and the response data structures and response codes specified in table 6.1.5.2.3.1-1.

Table 6.1.5.2.3.1-2: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MonitorUpdateResult | M | 1 | The MonitorUpdateResult shall contain the monitoring revocation results. |

Table 6.1.5.2.3.1-3: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [5] also apply. |  |  |  |  |

#### 6.1.5.3	MatchInformation

##### 6.1.5.3.1	Description

The POST method is used by the 5G DDNMF to report an observed matching result to a NF service consumer (e.g. 5G DDNMF).

##### 6.1.5.3.2	Target URI

The Callback URI "{matchInfoCallbackRef}" shall be used with the callback URI variables defined in table 6.1.5.3.2-1.

Table 6.1.5.3.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| matchInfoCallbackRef | String formatted as URI with the Callback Uri |

##### 6.1.5.3.3	Standard Methods

###### 6.1.5.3.3.1	POST

This method shall support the request data structures specified in table 6.1.5.3.3.1-1 and the response data structures and response codes specified in table 6.1.5.3.3.1-1.

Table 6.1.5.3.3.1-2: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MatchInformation | M | 1 | The MatchInformation shall contain a matching result of the corresponding operation. |

Table 6.1.5.3.3.1-3: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [5] also apply. |  |  |  |  |

### 6.1.6	Data Model

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the N5g-ddnmf service based interface protocol.

Table 6.1.6.1-1: N5g-ddnmf specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AnnounceAuthReqData | 6.1.6.2.2 | Represents Data used to request the authorization to announce for a UE |  |
| AnnounceAuthRespData | 6.1.6.2.3 | Represents the obtained Announce Authorize Data for a UE |  |
| AnnounceDiscDataForOpen | 6.1.6.2.4 | Represents Data for open discovery used to request the authorization to announce for a UE |  |
| AnnounceDiscDataForRestricted | 6.1.6.2.5 | Represents Data for restricted discovery used to request the authorization to announce for a UE |  |
| AnnounceUpdateData | 6.1.6.2.6 | Represents Announce Authorize Data to update. |  |
| MonitorAuthReqData | 6.1.6.2.7 | Represents Data used to request the authorization to monitor for a UE |  |
| MonitorAuthRespData | 6.1.6.2.8 | Represents the obtained Monitor Authorize Data for a UE |  |
| MonitorDiscDataForOpen | 6.1.6.2.9 | Represents Data for open discovery used to request the authorization to monitor for a UE |  |
| MonitorDiscDataForRestricted | 6.1.6.2.10 | Represents Data for restricted discovery used to request the authorization to monitor for a UE |  |
| MonitorAuthDataForOpen | 6.1.6.2.11 | Represents the obtained Announce Authorize Data for open discovery for a UE |  |
| MonitorAuthDataForRestricted | 6.1.6.2.12 | Represents the obtained Announce Authorize Data for restricted discovery for a UE |  |
| MonitorUpdateData | 6.1.6.2.13 | Represents Monitor Authorize Data to update. |  |
| DiscoveryAuthReqData | 6.1.6.2.14 | Represents Data used to request the authorization for a discoverer UE |  |
| DiscoveryAuthRespData | 6.1.6.2.15 | Represents the obtained authorization Data for a discoverer UE. |  |
| DiscDataForRestricted | 6.1.6.2.16 | Represents Data for restricted discovery used to request the authorization for a discoverer UE |  |
| AuthDataForRestricted | 6.1.6.2.17 | Represents obtained authorization Data for restricted discovery for a discoverer UE |  |
| MatchReportReqData | 6.1.6.2.18 | Represents the Match Report information |  |
| MatchReportRespData | 6.1.6.2.19 | Represents Match Report Acknowledgement |  |
| MonitorUpdateResult | 6.1.6.2.20 | Represents the monitoring revocation results. |  |
| MatchInformation | 6.1.6.2.21 | Represents a report including a matching result, and the information that can be used for charging purpose |  |
| MatchInfoForOpen | 6.1.6.2.22 | Represents a report including a matching result, and the information that can be used for charging purpose for the open discovery type. |  |
| MatchInfoForRestricted | 6.1.6.2.23 | Represents a report including a matching result, and the information that can be used for charging purpose for the restricted discovery type. |  |
| DiscoveryType | 6.1.6.3.3 | Represents Discovery Type for ProSe Service |  |
| AuthorizationResult | 6.1.6.3.4 | Represents Authorization Result Type for ProSe Service |  |
| RevocationResult | 6.1.6.3.5 | Represents the monitoring Revocation Result for ProSe Service |  |

Table 6.1.6.1-2 specifies data types re-used by the N5g-ddnmf service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the N<NF> service based interface.

Table 6.1.6.1-2: N5g-ddnmf re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| VarUeId | 3GPP TS 29.571 [16] | String represents the SUPI or GPSI. |  |
| DateTime | 3GPP TS 29.571 [16] | DataTime |  |
| PlmnId | 3GPP TS 29.571 [16] | Plmn ID |  |
| Supi | 3GPP TS 29.571 [16] | SUPI |  |
| Uri | 3GPP TS 29.571 [16] | URI |  |

#### 6.1.6.2	Structured data types

##### 6.1.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 6.1.6.2.2	Type: AnnounceAuthReqData

Table 6.1.6.2.2-1: Definition of type AnnounceAuthReqData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| discType | DiscoveryType | M | 1 | This IE shall contain the discovery type for ProSe Service. |  |
| openDiscData | AnnounceDiscDataForOpen | C | 0..1 | This IE shall contain the input data of announce authorisation for open discovery when present. If the value of discType is "OPEN", this IE shall be present. |  |
| restrictedDiscData | AnnounceDiscDataForRestricted | C | 0..1 | This IE shall contain the input data of announce authorisation for restricted discovery when present. If the value of discType is "RESTRICTED", this IE shall be present. |  |

##### 6.1.6.2.3	Type: AnnounceAuthRespData

Table 6.1.6.2.3-1: Definition of type AnnounceAuthRespData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| authResult | AuthorizationResult | M | 1 | This IE shall contain the authorization result. |  |

##### 6.1.6.2.4	Type: AnnounceDiscDataForOpen

Table 6.1.6.2.4-1: Definition of type AnnounceDiscDataForOpen

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| proseAppId | ProseApplicationId | M | 1 | This IE shall contain the ProSe Application ID. |  |
| proseAppCode | ProseApplicationCode | M | 1 | This IE shall contain the ProSe Application Code |  |
| validityTimer | DateTime | M | 1 | This IE shall contain validity timer up to which the ProSe Application Code is going to expire. |  |
| metaData | MetaData | O | 0..1 | This IE shall contain the metadata when present. |  |

##### 6.1.6.2.5	Type: AnnounceDiscDataForRestricted

Table 6.1.6.2.5-1: Definition of type AnnounceDiscDataForRestricted

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| rpauid | Rpauid | M | 1 | This IE shall contain the RPAUID. |  |
| appId | ApplicationId | M | 1 | This IE shall contain the Application ID |  |
| proseRestrictedCode | ProseRestrictedCode | C | 0..1 | This IE shall contain the ProSe Restricted Code when present. (NOTE) |  |
| proseRestrictedPrefix | ProseRestrictedPrefix | C | 0..1 | This IE shall contain the ProSe Restricted Prefix when present. (NOTE) |  |
| metaData | MetaData | O | 0..1 | This IE shall contain the metadata when present. |  |
| codeSuffixPool | RestrictedCodeSuffixPool | O | 0..1 | This IE shall contain the Restricted Code Suffix pool when present. |  |
| NOTE: Either attribute proseRestrictedCode or attribute proseRestrictedPrefix shall be present. |  |  |  |  |  |

##### 6.1.6.2.6	Type: AnnounceUpdateData

Table 6.1.6.2.6-1: Definition of type AnnounceUpdateData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| discType | DiscoveryType | M | 1 | This IE shall contain the discovery type for ProSe Service, only value "OPEN" is allowed. |  |
| validityTime | DateTime | M | 1 | This IE shall contain validity timer up to which the ProSe Application Code is going to expire. If the value sets to a full zero (i.e., 0000-00-00T00:00:00), it indicates to revoke the authorization for the announcing in the PLMN. |  |
| proseAppCode | ProseApplicationCode | C | 0..1 | This IE shall contain the ProSe Application Code when present. If the ProSe Application Code is changed, this IE shall be present. If the value of attribute is a full zero, this IE shall be absent. |  |

##### 6.1.6.2.7	Type: MonitorAuthReqData

Table 6.1.6.2.7-1: Definition of type MonitorAuthReqData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| discType | DiscoveryType | M | 1 | This IE shall contain the discovery type for ProSe Service. |  |
| openDiscData | MonitorDiscDataForOpen | C | 0..1 | This IE shall contain the input data of monitor authorisation for open discovery when present. If the value of discType is "OPEN", this IE shall be present. |  |
| restrictedDiscData | MonitorDiscDataForRestricted | C | 0..1 | This IE shall contain the input data of monitor authorisation for restricted discovery when present. If the value of discType is "RESTRICTED", this IE shall be present. |  |

##### 6.1.6.2.8	Type: MonitorAuthRespData

Table 6.1.6.2.8-1: Definition of type MonitorAuthRespData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| authResult | AuthorizationResult | M | 1 | This IE shall contain the authorization result. |  |
| authDataOpen | MonitorAuthDataForOpen | C | 0..1 | This IE shall contain a list of the ProSe Application Codes when present. If the authResult is "OPEN", this IE shall be present. |  |
| authDataRestricted | MonitorAuthDataForRestricted | C | 0..1 | This IE shall contain the Prose Application Prefix when present. If the authResult is "OPEN", this IE shall be present. |  |

##### 6.1.6.2.9	Type: MonitorDiscDataForOpen

Table 6.1.6.2.9-1: Definition of type MonitorDiscDataForOpen

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| proseAppIdNames | array(ProseApplicationIdName) | M | 1..N | This IE shall contain a list of ProSe Application ID names. |  |

##### 6.1.6.2.10	Type: MonitorDiscDataForRestricted

Table 6.1.6.2.10-1: Definition of type MonitorDiscDataForRestricted

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| rpauid | Rpauid | M | 1 | This IE shall contain the RPAUID. |  |
| targetPduid | Pduid | M | 1 | This IE shall contain the Target PDUID. |  |
| appId | ApplicationId | M | 1 | This IE shall contain the Application ID |  |
| targetRpauid | Rpauid | M | 1 | This IE shall contain the Target RPAUID. |  |

##### 6.1.6.2.11	Type: MonitorAuthDataForOpen

Table 6.1.6.2.11-1: Definition of type MonitorAuthDataForOpen

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| proseAppCodes | array(ProseApplicationCode) | C | 1..N | This IE shall contain a list of the ProSe Application Codes. (NOTE) |  |
| proseAppPrefix | ProseApplicationPrefix | C | 0..1 | This IE shall contain the Prose Application Prefix. (NOTE) |  |
| proseAppMasks | array(ProseApplicationMask) | M | 1..N | This IE shall contain a list of the ProSe Application Masks. |  |
| ttl | integer | M | 1 | This IE shall contain the TTL. If the value sets to zero, it indicates to revoke the authorization for the monitoring in the PLMN. |  |
| NOTE: Either attribute proseAppCodes or proseAppPrefix proseRestrictedPrefix shall be present if the discovery type is open. |  |  |  |  |  |

##### 6.1.6.2.12	Type: MonitorAuthDataForRestricted

Table 6.1.6.2.12-1: Definition of type MonitorAuthDataForRestricted

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| proseRestrictedCode | ProseRestrictedCode | M | 1 | This IE shall contain the ProSe Restricted Code. |  |
| validityTime | DateTime | M | 1 | This IE shall contain validity timer up to which the ProSe Application Code is going to expire. |  |

##### 6.1.6.2.13	Type: MonitorUpdateData

Table 6.1.6.2.13-1: Definition of type MonitorUpdateData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| discType | DiscoveryType | M | 1 | This IE shall contain the discovery type for ProSe Service. |  |
| proseAppIdName | ProseApplicationIdName | M | 1 | This IE shall contain the ProSe Application ID Name. |  |
| ttl | integer | M | 1 | This IE shall contain the TTL. |  |
| monitorUpdateResultCallbackRef | Uri | O | 0..1 | A URI provided by 5G DDNMF to receive (implicitly subscribed) notifications on the monitoring revocation results. |  |

##### 6.1.6.2.14	Type: DiscoveryAuthReqData

Table 6.1.6.2.14-1: Definition of type DiscoveryAuthReqData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| discType | DiscoveryType | M | 1 | This IE shall contain the discovery type for ProSe Service. Only value "RESTRICTED" is allowed. |  |
| restrictedDiscData | DiscDataForRestricted | C | 0..1 | This IE shall contain the input data of authorisation for restricted discovery when present. If the value of discType is "RESTRICTED", this IE shall be present. |  |

##### 6.1.6.2.15	Type: DiscoveryAuthRespData

Table 6.1.6.2.15-1: Definition of type DiscoveryAuthRespData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| authResult | AuthorizationResult | M | 1 | This IE shall contain the authorization result. |  |
| authDataRestricted | AuthDataForRestricted | C | 0..1 | This IE shall contain the authorized data for restricted discovery when present. If the discovery type in request is "RESTRICTED", this IE shall be present. |  |

##### 6.1.6.2.16	Type: DiscDataForRestricted

Table 6.1.6.2.16-1: Definition of type MonitorDiscDataForRestricted

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| rpauid | Rpauid | M | 1 | This IE shall contain the RPAUID. |  |
| targetPduid | Pduid | M | 1 | This IE shall contain the Target PDUID. |  |
| appId | ApplicationId | M | 1 | This IE shall contain the Application ID |  |
| targetRpauid | Rpauid | M | 1 | This IE shall contain the Target RPAUID. |  |

##### 6.1.6.2.17	Type: AuthDataForRestricted

Table 6.1.6.2.17-1: Definition of type AuthDataForRestricted

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| proseQueryCodes | array(ProseQueryCode) | M | 1..N | This IE shall contain a list of the authorised ProSe Restricted Code. |  |
| proseRespCode | ProseResponseCode | M | 1 | This IE shall contain the authorised ProSe Respond Code. |  |
| validityTime | DateTime | M | 1 | This IE shall contain validity timer up to which the ProSe Application Code is going to expire. |  |

##### 6.1.6.2.18	Type: MatchReportReqData

Table 6.1.6.2.18-1: Definition of type MatchReportReqData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| discType | DiscoveryType | M | 1 | This IE shall contain the discovery type for ProSe Service. Only value "OPEN" is allowed. |  |
| proseAppCodes | array(ProseApplicationCode) | C | 1..N | This IE shall contain a list of the ProSe Application Codes when present. If the value of DiscoveryType is "OPEN", this IE shall be present. |  |
| moniteredPlmnId | PlmnId | C | 0..1 | This IE shall contain the Monitored PLMN ID when present. |  |

##### 6.1.6.2.19	Type: MatchReportRespData

Table 6.1.6.2.19-1: Definition of type MatchReportRespData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| proseAppIdNames | array(ProseApplicationIdName) | C | 1..N | This IE shall contain a list of ProSe Application ID names when present. If the discovery type in the request is "OPEN", this IE shall be present. |  |
| validityTime | DateTime | C | 0..1 | This IE shall contain validity timer up to which the ProSe Application Code is going to expire when present. If the discovery type in the request is "OPEN", this IE shall be present. |  |
| metaData | MetaData | O | 0..1 | This IE shall contain the metadata when present. |  |
| metaDataIndexMasks | array(MetaDataIndexMask) | O | 1..N | This IE shall contain a list of the Meta Data Index Masks when present. |  |

##### 6.1.6.2.20	Type: MonitorUpdateResult

Table 6.1.6.2.20-1: Definition of type MonitorUpdateResult

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| discType | DiscoveryType | M | 1 | This IE shall contain the discovery type for ProSe Service. Only value "RESTRICTED" is allowed. |  |
| proseRestrictedCode | ProseRestrictedCode | M | 1 | This IE shall contain the ProSe Restricted Code. |  |
| appId | ApplicationId | M | 1 | This IE shall contain the Application ID |  |
| bannedRpauid | Rpauid | M | 1 | This IE shall contain the Banned RPAUID. |  |
| bannedPduid | Pduid | M | 1 | This IE shall contain the Banned PDUID. |  |
| revocationResult | RevocationResult | M | 1 | This IE shall contain the monitoring revocation results. |  |

##### 6.1.6.2.21	Type: MatchInformation

Table 6.1.6.2.21-1: Definition of type MatchInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| discType | DiscoveryType | M | 1 | This IE shall contain the discovery type for ProSe Service. |  |
| openMatchInfoForOpen | MatchInfoForOpen | C | 0..1 | This IE shall contain the match report information for open discovery type when present. If the value of discType is "OPEN", this IE shall be present. |  |
| restrictedMatchInfo | MatchInfoForRestricted | C | 0..1 | This IE shall contain the match report information for restricted discovery when present. If the value of discType is "RESTRICTED", this IE shall be present. |  |

##### 6.1.6.2.22	Type: MatchInfoForOpen

Table 6.1.6.2.22-1: Definition of type MatchInfoForOpen

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supi | Supi | M | 1 | This IE shall contain the SUPI of UE. |  |
| appId | array(ApplicationId) | M | 1 | This IE shall contain a list of the Application IDs. |  |

##### 6.1.6.2.23	Type: MatchInfoForRestricted

Table 6.1.6.2.23-1: Definition of type MatchInfoForRestricted

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supi | Supi | M | 1 | This IE shall contain the SUPI of UE. |  |
| rpauid | Rpauid | M | 1 | This IE shall contain the RPAUID. |  |
| targetPduid | Pduid | M | 1 | This IE shall contain the Target PDUID. |  |
| proseRestrictedCode | ProseRestrictedCode | M | 1 | This IE shall contain the ProSe Restricted Code. |  |

#### 6.1.6.3	Simple data types and enumerations

##### 6.1.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 6.1.6.3.2	Simple data types

The simple data types defined in table 6.1.6.3.2-1 shall be supported.

Table 6.1.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
| DiscoveryEntryId | string | Discovery Entry ID |  |
| ProseApplicationId | string | ProSe Application ID |  |
| ProseApplicationCode | string | ProSe Application Code |  |
| Rpauid | string | RPAUID |  |
| ApplicationId | string | Application ID |  |
| ProseRestrictedCode | string | ProSe Restricted Code |  |
| ProseRestrictedPrefix | string | ProSe Restricted Prefix |  |
| MetaData | string | metadata |  |
| RestrictedCodeSuffixPool | string | Restricted Code Suffix pool |  |
| ProseApplicationIdName | string | ProSe Application ID name |  |
| Pduid | string | PDUID |  |
| ProseApplicationPrefix | string | Prose Application Prefix |  |
| ProseApplicationMask | string | Prose Application Mask |  |
| ProseQueryCode | string | ProSe Query Code |  |
| ProseResponseCode | string | ProSe Response Code |  |
| MetaDataIndexMask | string | Meta Data Index Mask |  |

##### 6.1.6.3.3	Enumeration: DiscoveryType

The enumeration DiscoveryType represents Discovery Type for ProSe Service. It shall comply with the provisions defined in table 6.1.5.3.3-1.

Table 6.1.6.3.3-1: Enumeration DiscoveryType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "OPEN" | Discovery type is "open". |  |
| "RESTRICTED" | Discovery type is "restricted". |  |

##### 6.1.6.3.4	Enumeration: AuthorizationResult

The enumeration AuthorizationResult represents Authorization Result Type for ProSe Service. It shall comply with the provisions defined in table 6.1.5.3.4-1.

Table 6.1.6.3.4-1: Enumeration DiscoveryType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "SUCCESSFUL" | The authorisation is successful. |  |

##### 6.1.6.3.5	Enumeration: RevocationResult

The enumeration AuthorizationResult represents the monitoring Revocation Result for ProSe Service. It shall comply with the provisions defined in table 6.1.5.3.5-1.

Table 6.1.6.3.5-1: Enumeration RevocationResult

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "SUCCESSFUL" | The Monitoring Revocation is successful. |  |
| "FAILED" | The Monitoring Revocation is failed. |  |

#### 6.1.6.4	Data types describing alternative data types or combinations of data types

None.

#### 6.1.6.5	Binary data

None.

### 6.1.7	Error Handling

#### 6.1.7.1	General

For the N5g-ddnmf_Discovery API, HTTP error responses shall be supported as specified in clause 4.8 of 3GPP TS 29.501 [6]. Protocol errors and application errors specified in table 5.2.7.2-1 of 3GPP TS 29.500 [5] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.2.7.1-1 of 3GPP TS 29.500 [5].

In addition, the requirements in the following clauses are applicable for the N5g-ddnmf_Discovery API.

#### 6.1.7.2	Protocol Errors

Protocol errors handling shall be supported as specified in clause 5.2.7 of 3GPP TS 29.500 [5].

#### 6.1.7.3	Application Errors

The application errors defined for the N5g-ddnmf_Discovery service are listed in Table 6.1.7.3-1.

Table 6.1.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |

## 6.1.8	Feature negotiation

The optional features in table 6.1.8-1 are defined for the <API name> API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [5].

Table 6.1.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

## 6.1.9	Security

As indicated in 3GPP TS 33.501 [11] and 3GPP TS 29.500 [5], the access to the N5g-ddnmf_Discovery API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [12]), based on local configuration, using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [13]) plays the role of the authorization server.

If OAuth2 is used, an NF Service Consumer, prior to consuming services offered by the N5g-ddnmf_Discovery API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [13], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the N5g-ddnmf_Discovery service.

The N5g-ddnmf_Discovery API defines a single scope "n5g-ddnmf_discovery" for OAuth2 authorization (as specified in 3GPP TS 33.501 [11] for the entire service, and it does not define any additional scopes at resource or operation level.

Annex A (normative):
OpenAPI specification

## A.1	General

This Annex specifies the formal definition of the API(s) defined in the present specification. It consists of OpenAPI 3.0.0 specifications in YAML format.

This Annex takes precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API(s).

NOTE 1:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification files contained in this 3GPP Technical Specification are available on a Git-based repository that uses the GitLab software version control system (see 3GPP TS 29.501 [6] clause 5.3.1 and 3GPP TR 21.900 [14] clause 5B).

## A.2	N5g-ddnmf_Discovery API

openapi: 3.0.0

info:

title: N5g-ddnmf_Discovery API

version: 1.0.0-alpha.1

description: |

N5g-ddnmf_Discovery Service.

© 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.555 V0.1.0; 5G System; 5G Direct Discovery Name Management Services; Stage 3.

url: http://www.3gpp.org/ftp/Specs/archive/29_series/29.555/

servers:

- url: '{apiRoot}/n5g-ddnmf-disc/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- n5g-ddnmf-disc

paths:

/{ueId}/announce-authorize/{discEntryId}:

put:

summary: Obtain the authorization to announce for a UE

operationId: ObtainAnnounceAuth

tags:

- Obtain the authorization to announce for a UE

parameters:

- name: ueId

in: path

description: Identifier of the UE

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: discEntryId

in: path

description: Discovery Entry Id

required: true

schema:

$ref: '#/components/schemas/DiscoveryEntryId'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/AnnounceAuthReqData'

required: true

responses:

'201':

description: Created

content:

application/json:

schema:

$ref: '#/components/schemas/AnnounceAuthRespData'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/n5g-ddnmf-disc>/<apiVersion>/{ueId}/announce-authorize/{discEntryId}'

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

patch:

summary: Update the authorization for announcing for a UE

operationId: UpdateAnnounceAuth

tags:

- Update the authorization for announcing for a UE

parameters:

- name: ueId

in: path

description: Identifier of the UE

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: discEntryId

in: path

description: Discovery Entry Id

required: true

schema:

$ref: '#/components/schemas/DiscoveryEntryId'

requestBody:

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/AnnounceUpdateData'

required: true

responses:

'204':

description: Expected response to a valid request

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/{ueId}/monitor-authorize/{discEntryId}:

put:

summary: Obtain the authorization to monitor for a UE

operationId: ObtainMonitorAuth

tags:

- Obtain the authorization to monitor for a UE

parameters:

- name: ueId

in: path

description: Identifier of the UE

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: discEntryId

in: path

description: Discovery Entry Id

required: true

schema:

$ref: '#/components/schemas/DiscoveryEntryId'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/MonitorAuthReqData'

required: true

responses:

'201':

description: Created

content:

application/json:

schema:

$ref: '#/components/schemas/MonitorAuthRespData'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/n5g-ddnmf-disc>/<apiVersion>/{ueId}/monitor-authorize/{discEntryId}'

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

patch:

summary: Update the authorization for monitoring for a UE

operationId: UpdateMonitorAuth

tags:

- Update the authorization for monitoring for a UE

parameters:

- name: ueId

in: path

description: Identifier of the UE

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: discEntryId

in: path

description: Discovery Entry Id

required: true

schema:

$ref: '#/components/schemas/DiscoveryEntryId'

requestBody:

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/MonitorUpdateData'

required: true

responses:

'204':

description: Expected response to a valid request

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

callbacks:

MonitorUpdateResult:

'{request.body#/monitorUpdateResultCallbackRef}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MonitorUpdateResult'

responses:

'204':

description: Successful Notification response

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/{ueId}/discovery-authorize/{discEntryId}:

put:

summary: Obtain the authorization from the 5G DDNMF for a discoverer UE in the PLMN to operate Model B restricted discovery

operationId: ObtainDiscAuth

tags:

- Obtain the authorization for a discoverer UE

parameters:

- name: ueId

in: path

description: Identifier of the UE

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: discEntryId

in: path

description: Discovery Entry Id

required: true

schema:

$ref: '#/components/schemas/DiscoveryEntryId'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/DiscoveryAuthReqData'

required: true

responses:

'201':

description: Created

content:

application/json:

schema:

$ref: '#/components/schemas/DiscoveryAuthRespData'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/n5g-ddnmf-disc>/<apiVersion>/{ueId}/discovery-authorize/{discEntryId}'

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/{ueId}/match-report:

post:

summary: Obtain the information about the indicated discovery code from the 5G DDNMF

operationId: MatchReport

tags:

- Obtain the information about the indicated discovery code

parameters:

- name: ueId

in: path

description: Identifier of the UE

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/MatchReportReqData'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/MatchReportRespData'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

n5g-ddnmf-disc: Access to the N5g-ddnmf_Discovery API

schemas:

# COMPLEX TYPES:

AnnounceAuthReqData:

type: object

description: Represents Data used to request the authorization to announce for a UE

required:

- discType

properties:

discType:

$ref: '#/components/schemas/DiscoveryType'

openDiscData:

$ref: '#/components/schemas/AnnounceDiscDataForOpen'

restrictedDiscData:

$ref: '#/components/schemas/AnnounceDiscDataForRestricted'

AnnounceAuthRespData:

type: object

description: Represents the obtained Announce Authorize Data for a UE

required:

- authResult

properties:

authResult:

$ref: '#/components/schemas/AuthorizationResult'

AnnounceDiscDataForOpen:

type: object

description: Represents Data for open discovery used to request the authorization to announce for a UE

required:

- proseAppId

- proseAppCode

- validityTime

properties:

proseAppId:

$ref: '#/components/schemas/ProseApplicationId'

proseAppCode:

$ref: '#/components/schemas/ProseApplicationCode'

validityTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

metaData:

$ref: '#/components/schemas/MetaData'

AnnounceDiscDataForRestricted:

type: object

description: Represents Data for restricted discovery used to request the authorization to announce for a UE

required:

- rpauid

- appId

properties:

rpauid:

$ref: '#/components/schemas/Rpauid'

appId:

$ref: '#/components/schemas/ApplicationId'

proseRestrictedCode:

$ref: '#/components/schemas/ProseRestrictedCode'

proseRestrictedPrefix:

$ref: '#/components/schemas/ProseRestrictedPrefix'

metaData:

$ref: '#/components/schemas/MetaData'

codeSuffixPool:

$ref: '#/components/schemas/RestrictedCodeSuffixPool'

AnnounceUpdateData:

type: object

description: Represents Announce Authorize Data to update

required:

- discType

- validityTime

properties:

discType:

$ref: '#/components/schemas/DiscoveryType'

validityTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

proseAppCode:

$ref: '#/components/schemas/ProseApplicationCode'

MonitorAuthReqData:

type: object

description: Represents Data used to request the authorization to monitor for a UE

required:

- discType

properties:

discType:

$ref: '#/components/schemas/DiscoveryType'

openDiscData:

$ref: '#/components/schemas/MonitorDiscDataForOpen'

restrictedDiscData:

$ref: '#/components/schemas/MonitorDiscDataForRestricted'

MonitorAuthRespData:

type: object

description: Represents the obtained Monitor Authorize Data for a UE

required:

- authResult

properties:

authResult:

$ref: '#/components/schemas/AuthorizationResult'

authDataOpen:

$ref: '#/components/schemas/MonitorAuthDataForOpen'

authDataRestricted:

$ref: '#/components/schemas/MonitorAuthDataForRestricted'

MonitorDiscDataForOpen:

type: object

description: Represents Data for open discovery used to request the authorization to monitor for a UE

required:

- proseAppIdNames

properties:

proseAppIdNames:

type: array

items:

$ref: '#/components/schemas/ProseApplicationIdName'

minItems: 1

MonitorDiscDataForRestricted:

type: object

description: Represents Data for restricted discovery used to request the authorization to monitor for a UE

required:

- rpauid

- targetPduid

- appId

- targetRpauid

properties:

rpauid:

$ref: '#/components/schemas/Rpauid'

targetPduid:

$ref: '#/components/schemas/Pduid'

appId:

$ref: '#/components/schemas/ApplicationId'

targetRpauid:

$ref: '#/components/schemas/Rpauid'

MonitorAuthDataForOpen:

type: object

description: Represents the obtained Announce Authorize Data for open discovery for a UE

required:

- proseAppMasks

- ttl

properties:

proseAppCodes:

type: array

items:

$ref: '#/components/schemas/ProseApplicationCode'

minItems: 1

proseAppPrefix:

$ref: '#/components/schemas/ProseApplicationPrefix'

proseAppMasks:

type: array

items:

$ref: '#/components/schemas/ProseApplicationMask'

minItems: 1

ttl:

type: integer

MonitorAuthDataForRestricted:

type: object

description: Represents the obtained Announce Authorize Data for restricted discovery for a UE

required:

- proseRestrictedCode

- validityTime

properties:

proseRestrictedCode:

$ref: '#/components/schemas/ProseRestrictedCode'

validityTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

MonitorUpdateData:

type: object

description: Represents Monitor Authorize Data to update.

required:

- discType

- proseAppIdName

- ttl

properties:

discType:

$ref: '#/components/schemas/DiscoveryType'

proseAppIdName:

$ref: '#/components/schemas/ProseApplicationIdName'

ttl:

type: integer

monitorUpdateResultCallbackRef:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

DiscoveryAuthReqData:

type: object

description: Represents Data used to request the authorization for a discoverer UE.

required:

- discType

properties:

discType:

$ref: '#/components/schemas/DiscoveryType'

restrictedDiscData:

$ref: '#/components/schemas/DiscDataForRestricted'

DiscoveryAuthRespData:

type: object

description: Represents the obtained authorization Data for a discoverer UE

required:

- authResult

properties:

authResult:

$ref: '#/components/schemas/AuthorizationResult'

authDataRestricted:

$ref: '#/components/schemas/AuthDataForRestricted'

DiscDataForRestricted:

type: object

description: Represents Data for restricted discovery used to request the authorization for a discoverer UE

required:

- rpauid

- targetPduid

- appId

- targetRpauid

properties:

rpauid:

$ref: '#/components/schemas/Rpauid'

targetPduid:

$ref: '#/components/schemas/Pduid'

appId:

$ref: '#/components/schemas/ApplicationId'

targetRpauid:

$ref: '#/components/schemas/Rpauid'

AuthDataForRestricted:

type: object

description: Represents obtained authorization Data for restricted discovery for a discoverer UE

required:

- proseQueryCodes

- proseRespCode

- validityTime

properties:

proseQueryCodes:

items:

$ref: '#/components/schemas/ProseQueryCode'

minItems: 1

proseRespCode:

$ref: '#/components/schemas/ProseResponseCode'

validityTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

MatchReportReqData:

type: object

description: Represents the Match Report information

required:

- discType

properties:

discType:

$ref: '#/components/schemas/DiscoveryType'

proseAppCodes:

items:

$ref: '#/components/schemas/ProseApplicationCode'

minItems: 1

moniteredPlmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

MatchReportRespData:

type: object

description: Represents Match Report Acknowledgement

properties:

proseAppIdNames:

items:

$ref: '#/components/schemas/ProseApplicationIdName'

minItems: 1

validityTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

metaData:

$ref: '#/components/schemas/MetaData'

metaDataIndexMasks:

items:

$ref: '#/components/schemas/MetaDataIndexMask'

minItems: 1

MonitorUpdateResult:

type: object

description: Represents the monitoring revocation results.

required:

- discType

- proseRestrictedCode

- appId

- bannedRpauid

- bannedPduid

- revocationResult

properties:

discType:

$ref: '#/components/schemas/DiscoveryType'

proseRestrictedCode:

$ref: '#/components/schemas/ProseRestrictedCode'

appId:

$ref: '#/components/schemas/ApplicationId'

bannedRpauid:

$ref: '#/components/schemas/Rpauid'

bannedPduid:

$ref: '#/components/schemas/Pduid'

revocationResult:

$ref: '#/components/schemas/RevocationResult'

MatchInformation:

type: object

description: Represents a report including a matching result, and the information that can be used for charging purpose.

required:

- discType

properties:

discType:

$ref: '#/components/schemas/DiscoveryType'

openMatchInfoForOpen:

$ref: '#/components/schemas/MatchInfoForOpen'

restrictedMatchInfo:

$ref: '#/components/schemas/MatchInfoForRestricted'

MatchInfoForOpen:

type: object

description: Represents a report including a matching result, and the information that can be used for charging purpose for the open discovery type.

required:

- supi

- appId

properties:

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

appId:

items:

$ref: '#/components/schemas/ApplicationId'

minItems: 1

MatchInfoForRestricted:

type: object

description: Represents a report including a matching result, and the information that can be used for charging purpose for the restricted discovery type.

required:

- supi

- rpauid

- targetPduid

- proseRestrictedCode

properties:

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

rpauid:

$ref: '#/components/schemas/Rpauid'

targetPduid:

$ref: '#/components/schemas/Pduid'

proseRestrictedCode:

$ref: '#/components/schemas/ProseRestrictedCode'

# SIMPLE TYPES:

DiscoveryEntryId:

description: Contains the Discovery Entry ID.

type: string

ProseApplicationId:

description: Contains the ProSe Application ID.

type: string

ProseApplicationCode:

description: Contains the ProSe Application Code.

type: string

Rpauid:

description: Contains the RPAUID.

type: string

ApplicationId:

description: Contains the Application ID.

type: string

ProseRestrictedCode:

description: Contains the ProSe Restricted Code.

type: string

ProseRestrictedPrefix:

description: Contains the ProSe Restricted Prefix.

type: string

MetaData:

description: Contains the metadata.

type: string

RestrictedCodeSuffixPool:

description: Contains the Restricted Code Suffix pool.

type: string

ProseApplicationIdName:

description: Contains the ProSe Application ID name.

type: string

Pduid:

description: Contains the PDUID.

type: string

ProseApplicationPrefix:

description: Contains the Prose Application Prefix.

type: string

ProseApplicationMask:

description: Contains the Prose Application Mask.

type: string

ProseQueryCode:

description: Contains the ProSe Query Code.

type: string

ProseResponseCode:

description: Contains the ProSe Response Code.

type: string

MetaDataIndexMask:

description: Contains the Meta Data Index Mask.

type: string

# ENUMS:

DiscoveryType:

anyOf:

- type: string

enum:

- OPEN

- RESTRICTED

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Possible values are

- OPEN: Discovery type is "open".

- RESTRICTED: Discovery type is "restricted".

AuthorizationResult:

anyOf:

- type: string

enum:

- SUCCESSFUL

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Possible values are

- SUCCESSFUL: The authorisation is successful.

RevocationResult:

anyOf:

- type: string

enum:

- SUCCESSFUL

- FAILED

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Possible values are

- SUCCESSFUL: The Monitoring Revocation is successful.

- FAILED: The Monitoring Revocation is failed.

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-04 | C4#103-e | C4-212597 |  |  |  | Implementation of pCRs agreed in CT4 # 103-e including C4-212149, C4-212151, C4-212455. | 0.1.0 |
| 2021-06 | C4#104-e | C4-213528 |  |  |  | Implementation of pCRs agreed in CT4#104-e including C4-213188, C4-213207, C4-213338, C4-213340, C4-213341, C4-213342, C4-213455, C4-213457, C4-213463. | 0.2.0 |