| 3GPP TS 29.585 V0.2.0 (2023-09) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; 5G System (5GS); Session Management Function (SMF) / Centralized User Configuration (CUC) to Access Network Talker Listener (AN-TL) and Core Network Talker Listener (CN-TL) protocol aspects; Stage 3 (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	8

3.3	Abbreviations	8

4	General	8

4.1	Introduction	8

5	Elementary procedures between SMF/CUC and AN-TL and CN-TL	9

5.1	General	9

5.2	Procedures	9

5.2.1	SMF/CUC-initiated Get procedure	9

5.2.1.1	General	9

5.2.1.2	SMF/CUC-initiated Get procedure initiation	10

5.2.1.3	SMF/CUC-initiated Get procedure completion	10

5.2.1.4	Abnormal cases in the SMF/CUC	10

5.2.1.5	Abnormal cases in the AN-TL/CN-TL	11

5.2.2	SMF/CUC-initiated Set procedure	11

5.2.2.1	General	11

5.2.2.2	SMF/CUC-initiated Set procedure initiation	12

5.2.2.3	SMF/CUC-initiated Set procedure completion	13

5.2.2.4	Abnormal cases in the SMF/CUC	13

5.2.2.5	Abnormal cases in the AN-TL/CN-TL	13

6	Handling of unknown, unforeseen, and erroneous data	14

6.1	General	14

6.2	TBD	14

7	Messages and Message Format	14

7.1	Message format and encoding	14

7.1.1	Transmission Order and Bit Definitions	14

7.1.2	Message Format	14

7.1.2.1	General	14

7.1.2.2	Message Header	14

7.1.3	Information Elements	15

7.1.3.1	General	15

7.1.3.2	Presence Requirements of Information Elements	15

7.1.3.3	Grouped Information Elements	16

7.1.3.4	Information Element Type	16

7.2	Message Types	16

7.3	Messages	17

7.3.1	Get Request	17

7.3.1.1	Message definition	17

7.3.2	Get Response	17

7.3.2.1	Message definition	17

7.3.3	Set Request	17

7.3.3.1	Message definition	17

7.3.4	Set Response	19

7.3.4.1	Message definition	19

7.4	Error Handling	20

7.4.1	Protocol Errors	20

7.4.2	Message of Invalid Length	20

7.4.3	Unknown Message	20

7.4.4	Unexpected Message	20

7.4.5	Missing Information Elements	20

7.4.6	Semantically incorrect Information Element	21

7.4.7	Unknown or unexpected Information Element	21

7.4.8	Repeated Information Elements	21

8	Information elements coding	22

8.1	Information Elements Format	22

8.2	Information Element Types	23

8.2.1	General	23

8.2.2	Cause	24

8.2.3	Requested ES Parameters	25

8.2.4	End Station Interface	25

8.2.5	Interface Capabilities	26

8.2.6	TN Stream ID	26

8.2.7	Mask-and-match information	26

8.2.8	Destination MAC address	27

8.2.9	Source MAC address	27

8.2.10	VLAN Tag Info	27

8.2.11	IPv4 tuple	28

8.2.12	IPv6 tuple	28

8.2.13	Interface configuration	29

8.2.14	Interface Name	30

8.2.15	Gate Control Parameters	30

9	Timers	31

Annex A (informative): Change history	32

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the protocol of communication in the 5G System between:

-	the SMF/CUC and the (R)AN/AN-TL, and

-	the SMF/CUC and the UPF/CN-TL,

when the (R)AN and the UPF support the Talker and Listener functionality, for the support of Time Sensitive Networking (TSN) enabled Transport Network (TN) as specified in clauses 4.4.8, 5.28a and Annex M of 3GPP TS 23.501 [2] and Annex F of 3GPP TS 23.502 [3].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	IEEE Std 802.1Q-2022: "IEEE Standard for Local and Metropolitan Area Networks-Bridges and Bridged Networks".

[5]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[6]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane nodes".w

[7]	IEEE Std  802.1CBdb-2021: "IEEE Standard for Local and metropolitan area networks-Frame Replication and Elimination for Reliability".

[8]	IETF RFC 2474: "Definition of the Differentiated Services Field (DS Field) in the IPv4 and IPv6 Headers".

# 3	Definitions of terms, symbols and abbreviations

This clause and its three (sub) clauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Stream: A unidirectional flow of time-sensitive data from one source to one or more destinations, and at the highest level, one Talker end system to one or more Listener end systems.

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.501 [2] apply:

5G System

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GS	5G System

AN-TL	Access Network Talker Listener function

CN-TL	Core Network Talker Listener function

CNC	Centralized Network Configuration

CUC	Centralized User Configuration

ES	End Station

(R)AN	(Radio) Access Network

SMF	Session Management Function

TL	Talker Listener

TN	Transport Network

TSN	Time-Sensitive Networking

UPF	User Plane Function

# 4	General

## 4.1	Introduction

The 5GS supports interworking with IEEE 802.1 TSN deployed in the transport network. The interworking is applicable when the transport network deploys the fully centralized configuration model as defined in IEEE Std 802.1Q [4]. In this scenario, a TSN TN is deployed to realize the N3 interface between the (R)AN and the UPF. From the perspective of the TSN enabled TN, the (R)AN and the UPF act as End Stations (ES) of the TSN TN.

Figure 4.1-1 depicts the reference model for 5GS interworking with TSN enabled TN, with focus on the interfaces between the SMF/CUC and the (R)AN/AN-TL and between the SMF/CUC and the UPF/CN-TL which are in scope of this specification.

Figure 4.1-1: Reference Model – 5GS interworking with TSN enabled TN

If the (R)AN and the UPF support the TSN Talker and Listener (TL) functionality (i.e. they implement the AN-TL and CN-TL, respectively), the SMF/CUC can communicate with the AN-TL and CN-TL to:

-	retrieve the ES capabilities (list of interfaces and interface capabilities) of the AN-TL and CN-TL; and

-	configure TSN features in the ES (e.g. for stream identification, stream association with TSN resources, stream transformation, hold and buffer functionality) of the AN-TL and CN-TL.

The messages defined in this specification are exchanged between the SMF/CUC and the (R)AN/AN-TL and UPF/CN-TL via TL-Containers sent within NGAP Session Management signaling as specified in 3GPP TS 38.413 [5] and PFCP Session related signaling as specified in 3GPP TS 29.244 [6]. The related stage 2 call flows are specified in 3GPP TS 23.502 [3].

# 5	Elementary procedures between SMF/CUC and AN-TL and CN-TL

## 5.1	General

## 5.2	Procedures

### 5.2.1	SMF/CUC-initiated Get procedure

#### 5.2.1.1	General

The SMF/CUC may retrieve the following ES information of the AN-TL and CN-TL (see Table M.2-1 of 3GPP TS 23.501 [2]) during the establishment of a PDU session:

1)	ES interfaces, consisting of a list of one or more Interface IDs, where each Interface ID identifies a physical interface (distinct point of attachment) in the ES acting as Talker/Listener (see clause 46.2.3.3 of IEEE Std 802.1Q [4]); and/or

Editor's note: clause 5.28a.2 of TS 23.501 specifies "EndStationInterfaces: list of InterfaceIDs, one InterfaceID is associated with each Local F-TEID". It should be clarified: 
a)	why an Interface ID needs to be associated with each Local F-TEID, 
b)	what does this mean for a Talker for which there is no "local F-TEID" (the Talker sends packets over a GTP-U tunnel towards a remote F-TEID).  
c)	when the ES supports a single ES interface, whether this assumes that the ES needs to report "virtual" ES interfaces using a different Interface Name per F-TEID?

2)	Interface capabilities, describing the network capabilities of all the ES interfaces. The following capabilities may be signaled in this release of the specification:

-	Vlan Tag capability, indicating whether the ES interfaces support the ability to tag/untag frames using a Customer VLAN Tag (C-TAG) provided by the network (see clause 46.2.3.7.1 of of IEEE Std 802.1Q [4]);

-	Buffering capability, for a Talker, indicating the maximum possible buffer duration for a packet with the maximum size of an Ethernet packet (1522 Bytes) (see Table M.2-1 of 3GPP TS 23.501 [2]).

#### 5.2.1.2	SMF/CUC-initiated Get procedure initiation

In order to retrieve the ES parameters of the AN-TL or CN-TL, the SMF/CUC shall:

a)	encode the information about the requested ES parameters (End station interfaces and/or Interface capabilities) in the Requested ES Parameters IE in a GET REQUEST message;

b)	send the GET REQUEST message to the AN-TL or CN-TL as specified in 3GPP TS 23.502 [3]; and

c)	start the timer Tget.

Figure 5.2.1.2-1: SMF/CUC-initiated Get procedure

#### 5.2.1.3	SMF/CUC-initiated Get procedure completion

Upon receipt of the Get Request message, the AN-TL/CN-TL shall:

-	report the list of its interfaces in End Station Interfaces IE(s) of the Get Response message, if the ES ITF bit was set to 1 in the Requested ES Parameters IE in the Get Request message;

-	report the network capabilities of its interfaces in the Interface Capabilities IE, if the ITF CAP bit was set to 1 in the Requested ES Parameters IE in the Get Request message; and

-	send the Get Response message to the SMF/CUC as specified in 3GPP TS 23.502 [3].

#### 5.2.1.4	Abnormal cases in the SMF/CUC

The following abnormal cases can be identified:

a)	Tget expired.

The SMF/CUC should, on the first expiry of the timer Tget, retransmit the Get Request message and, if doing so, it shall reset and start timer Tget. This retransmission may be repeated a few times. On the expiry of timer at the last retransmission, the SMF/CUC shall abort the procedure.

#### 5.2.1.5	Abnormal cases in the AN-TL/CN-TL

The following abnormal cases can be identified:

a)	Transmission failure of the Get Response message indication (e.g. transient communication failure over N4).

The AN-TL/CN-TL shall not diagnose an error and consider the SMF/CUC-initiated port management procedure complete.

### 5.2.2	SMF/CUC-initiated Set procedure

#### 5.2.2.1	General

The SMF/CUC may configure the following ES information in the AN-TL and CN-TL, at the Talker and Listener, per TN stream of a PDU session (see Table M.2-1 of 3GPP TS 23.501 [2]), during the establishment or the modification of a PDU session:

Editor's note 1: TS 23.502 does not describe the use of set-request/response in the Service Request procedure. It is assumed this is because PDU sessions using TSN enabled TN are expected to have their UP connectivity always active.

1)	When configuring a new stream:

a)	TN stream ID uniquely identifying the stream configuration at the ES.

Editor's note 2: It is assumed that a single set request can be used to add, modify, and delete multiple streams of a same PDU session. Currently, a TN stream identifier is not listed in Table M.2-1 of TS 23.501. However, protocol-wise CT4 considers that it is useful information to identify the stream configuration when modifying or deleting the stream configuration at the AN-TL/CN-TL. It is also useful to allow reporting the TN stream(s) for which the requested stream configuration failed, assuming a single set request may be used to add, modify, or delete multiple streams of a same PDU session. SA2 needs to confirm that it is agreeable that the SMF/CUC signals a TN Stream identification (e.g. StreamID) in the Set Request for the aforementioned reasons.

b)	TN stream identification information, identifying the packets belonging to the new stream being configured. It shall either comprise:

-	a mask-and-match information, set with the destination GTP-U F-TEID of the GTP-U tunnel of the PDU session and with the QFI assigned to the TN stream; or

-	a Data Frame Specification, with the IPv4-tuple or IPv6-type set with the destination GTP-U IP address of the GTP-U tunnel of the TN stream of the PDU session.

NOTE 1:	The Data Frame Specification can be used to identify the packets of the TN stream if a separate GTP-U tunnel, with a distinct IP address (of the GTP-U F-TEID) is established for the TN stream.

Editor's note 3: SA2 should clarify why the Destination/Source MAC addresses and VLAN ID are also contained in DataFrameSpecification as parameters in the TL-Container in Table M.2-1 of TS 23.501.

c)	Interface configuration of the ES for the new TN stream, indicating the interface/port with which the new TN stream is to be associated.

d)	If stream transformation is supported by the AN-TL/CN-TL and required to be performed at the AN-TL/CN-TL, the Destination MAC address, Priority Code Point and/or VLAN ID (if the ES interface is VLAN capable) to be used for the stream transformation before the packet is transmitted via the interface/port with which the TN stream is associated.

2)	When modifying the configuration of an existing stream:

a)	TN stream ID uniquely identifying the stream configuration at the ES.

b)	Interface configuration of the ES for the TN stream, if the SMF/CUC selects an alternative port (Interface Name) to the same nearest bridge for the TN stream.

c)	new Time Aware Offset value as described in step 4.

NOTE 2:	The SMF/CUC can modify the Time Aware Offset of a TN stream configuration, optionally also selecting an alternative port (Interface Name) to the same nearest bridge for the TN stream, when a new Listener joins the stream, e.g. to meet the new Maximum Latency.

NOTE 3:	The TN stream identification information cannot change in this release of the specification since it is not supported to change the GTP-U F-TEID of a PDU session (or of a TN stream) using TSN enabled TN.

NOTE 4:	The following parameters of a TN stream cannot be modified: selection of a new ES interface with a different MAC address, Destination MAC address, Priority Code Point and VLAN ID.

Editor's note 4: The stream configuration modifications that should be supported in this release of the specification need to be confirmed by SA2.

3)	When deleting the configuration of an existing stream:

a)	TN stream ID uniquely identifying the stream's configuration at the ES.

NOTE 5:	The deletion of a stream configuration applies e.g., when a QoS flow is terminated during a PDU session modification procedure.

Editor's note 5: TS 23.502 describes the sending of Set-Request/Response during the PDU session establishment and PDU session modification procedures. It is not specified though how TN streams configured at the AN-TL and CN-TL are deleted when the PDU session is terminated (e.g. whether these TN streams should be deleted without explicit signaling from the SMF/CUC when the PDU session resources are released at the (R)AN or when the PFCP session related to the PDU session is terminated) and how the input parameters earlier received by the AN-TL/CN-TL to calculate the Gate Control Information should be updated when the interface/port(s) used by the streams of the PDU session that is terminated are also associated with streams from other PDU sessions.

4)	In all the above cases (1, 2 and 3):

a)	parameters to enable the Talker to calculate for each interface/port Gate Control Information, comprising the Interval, Max Frame Size and/or Time Aware Offset parameters of all the TN streams of all the PDU sessions mapped to the Interface/Port associated with the TN stream being added, modified or deleted.

Editor's note 6: In cases where multiple SMF/CUCs serve the PDU sessions established in a UPF or gNB, it is assumed that an ES (i.e. AN-TL or CN-TL) with its port(s) is always dedicated to a single SMF/CUC to avoid inconsistent gate control information.

Editor's note 7: If the AN-TL/CN-TL receives multiple Set Requests (adding, modifying, deleting TN streams) for the same or different PDU sessions, it is important that the TL processes the requests in the correct order to configure the final Gate Control information at the port(s)of the ES. To allow to send different set requests for TN streams of the same or different PDU sessions concurrently, it is assumed that the SMF/CUC must provide a sequencing mechanism for these parallel requests, which enables the TL to process the set requests in the correct order. The Sequence Number defined in the message header may be considered for that purpose.

The SMF/CUC may add, modify or delete the configurations of multiple TN streams of a same PDU session in a single Set procedure (i.e. Set Request).

NOTE 6:	As indicated in step 4 above, the SMF/CUC can modify the input parameters for the calculation of Gate Control Information, for one or more interface/port(s), for all the TN streams of all the PDU sessions associated with the interface/port(s), in a single Set Request.

#### 5.2.2.2	SMF/CUC-initiated Set procedure initiation

In order to configure the ES information in the AN-TL and CN-TL, the SMF/CUC shall:

a)	encode the information about the requested configuration in a Set Request message with:

-	an Add TN Stream Configuration IE, a Modify TN Stream Configuration IE or a Delete TN Stream Configuration IE, if it is requested to configure a new stream, modify the configuration of an existing stream, or delete the configuration of an existing TN stream, respectively, with the information described in clause 5.2.2.1;

-	one or more Add TN Stream Configuration IE(s), Modify TN Stream Configuration IE(s) or Delete TN Stream Configuration IE(s) may be included in the request, if more than one steam configuration (for the same PDU session) needs to be added, modified or deleted;

-	one or more Parameters for Gate Control Information Calculation IE(s), to enable the AN-TL/CN-TL to calculate gate control information for the Interface/Port(s) associated with TN streams being added, modified or deleted.

b)	send the Set Request message to the AN-TL or CN-TL as specified in 3GPP TS 23.502 [3]; and

c)	start the timer Tset.

Figure 5.2.2.2-1: SMF/CUC-initiated Set procedure

#### 5.2.2.3	SMF/CUC-initiated Set procedure completion

Upon receipt of the Set Request message, the AN-TL/CN-TL shall:

-	store and attempt to apply the new or modified TN stream configuration, for TN stream configurations requested to be added or modified;

-	delete the TN stream configuration, for TN stream configurations requested to be deleted;

-	replace any earlier received Parameters for Gate Control Information Calculation IE(s) for a given interface/port, if new parameters are received for certain interface/port(s), and calculate the Gate Control Information; and

-	send the Set Response message to the SMF/CUC as specified in 3GPP TS 23.502 [3].

#### 5.2.2.4	Abnormal cases in the SMF/CUC

FFS

#### 5.2.2.5	Abnormal cases in the AN-TL/CN-TL

The (R)AN/AN-TL and UPF/CN-TL shall return a Failure Code if it detects a mismatch between the Data Frame Specification and the GTP-U information received outside of the TL-Container, as specified in NOTE 13 of Table M.2-1 of 3GPP TS 23.501 [2].

NOTE:	This assumes that the AN-TL/CN-TL receives the GTP-U information of the PDU session from the (R)AN/UPF, respectively, by implementation specific means.

FFS

# 6	Handling of unknown, unforeseen, and erroneous data

Protocol error handling

## 6.1	General

## 6.2	TBD

# 7	Messages and Message Format

## 7.1	Message format and encoding

### 7.1.1	Transmission Order and Bit Definitions

Messages shall be transmitted in network octet order starting with octet 1 with the most significant bit sent first.

The most significant bit of an octet in a message is bit 8. If a field in a message spans over several octets, the most significant bit is bit 8 of the octet with the lowest number, unless specified otherwise.

### 7.1.2	Message Format

#### 7.1.2.1	General

The format of a message is depicted in Figure 7.1.2.1-1.

|  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octets | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | 1 to m | Message header |  |  |  |  |  |  |  |  |
|  | m+1 to n | Zero or more Information Element(s) |  |  |  |  |  |  |  |  |

Figure 7.1.2.1-1: Message Format

A message shall contain the message header and may contain subsequent information element(s) dependent on the type of message.

The encoding of the Information elements is specified in clause 8.1.

#### 7.1.2.2	Message Header

Messages use a fixed length header of 4 octets. Figure 7.1.2.2-1 illustrates the format of the Header.

|  |  | Bits |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets |  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| 1 |  | Version |  |  | Spare |  |  |  |  |
| 2 |  | Message Type |  |  |  |  |  |  |  |
| 3 |  | Sequence Number (1st Octet) |  |  |  |  |  |  |  |
| 4 |  | Sequence Number (2nd Octet) |  |  |  |  |  |  |  |

Figure 7.1.2.2-1: Message Header

The first octet of the header shall be used as follows:

-	Bit 1-5 are spare bits. The sending entity shall set it to "0" and the receiving entity shall ignore it.

-	Bits 6 to 8 represent the version of the protocol which shall be set to decimal vaue 1 ("001").

The usage of the fields in octets 2 - 4 of the header shall be as specified below.

-	Octet 2 represents the Message type field, which shall be set to the unique value for each type of message. Message type values are specified in Table 7.3-1 "Message types".

-	Octets 3 to 4 represent the Sequence Number field which enables to correlate a response with a request.

NOTE:	The SMF/CUC can send multiple Set Request messages to the AN-TL/CN-TL for the same PDU session or different PDU sessions in parallel. The response message contains the same sequence number as the corresponding request message.

### 7.1.3	Information Elements

#### 7.1.3.1	General

The format of Information Elements are defined in clauses 8.1 and 8.2.

#### 7.1.3.2	Presence Requirements of Information Elements

IEs within messages shall be specified with one of the following presence requirements:

-	Mandatory: this means that the IE shall be included by the sending entity, and that the receiver diagnoses a "Mandatory IE missing" error when detecting that the IE is not present. A response including a "Mandatory IE missing" cause, shall include the type of the missing IE.

-	Conditional: this means that:

-	the IE shall be included by sending entity if the conditions specified are met;

-	the receiver shall check the conditions as specified in the corresponding message type description, based on the parameter combination in the message and/or on the state of the receiving node, to infer if a conditional IE shall be expected. Only if a receiver has sufficient information, if a conditional IE, which is necessary for the receiving entity to complete the procedure, is missing, then the receiver shall abort the procedure.

-	Optional: this means that:

-	the IE shall be included as a service option. Therefore, the IE may be included or not in a message. The handling of an absent optional IE, or an erroneous optional IE is specified in clause 7.4.

For conditional IEs, the clause describing the message explicitly defines the conditions under which the inclusion of each IE becomes mandatory or optional for that particular message. These conditions shall be defined so that the presence of a conditional IE only becomes mandatory if it is critical for the receiving entity.

For grouped IEs, the presence requirement of the embedded IE shall follow the rules:

-	If the grouped IE is Mandatory within a given message: the presence requirements of individual embedded IEs are as stated within the Mandatory grouped IE for the given message;

-	if the grouped IE is Conditional within a given message: if the embedded IE in the grouped IE is Mandatory or Conditional, this embedded IE is viewed as Conditional IE by the receiver. If the embedded IE in the grouped IE is Conditional-Optional, this embedded IE is viewed as Optional IE by the receiver. If the embedded IE in the grouped IE is Optional, this embedded IE is viewed as Optional IE by the receiver;

-	if the grouped IE is Conditional-Optional within a given message: if the embedded IE in the grouped IE is Mandatory or Conditional, this embedded IE is viewed as Conditional-Optional IE by the receiver. If the embedded IE in the grouped IE is Conditional-Optional, this embedded IE is viewed as Optional IE by the receiver. If the embedded IE in the grouped IE is Optional, this embedded IE is viewed as Optional IE by the receiver;

-	if the grouped IE is Optional within a given message: all embedded IEs in the grouped IE are viewed as Optional IEs by the receiver.

In all of the above cases, appropriate error handling as described in clause 7.4 shall be applied for protocol errors of the embedded IEs.

Only the Cause IE at message level shall be included in the response if the Cause contains a value that indicates that the request is not accepted, regardless of whether there are other mandatory or conditional IEs defined for a given response message.

#### 7.1.3.3	Grouped Information Elements

A Grouped IE is an IE which may contain other IEs.

Grouped IEs have a length value in the TLV encoding, which includes the added length of all the embedded IEs. Overall coding of a grouped IE with 4 octets long IE header is defined in clause 8.2. Each IE within a grouped IE also shall also contain 4 octets long IE header.

Grouped IEs are not marked by any flag or limited to a specific range of IE type values. The clause describing an IE in this specification shall explicitly state if it is a Grouped IE.

NOTE:	Each entry into each Grouped IE creates a new scope level. Exit from the grouped IE closes the scope level. The message level is the top most scope.

If more than one grouped IEs of the same type, but for a different purpose are sent within the same message level, these IEs shall have different IE types.

If more than one grouped IEs of the same type and for the same purpose are sent within the same message level, these IEs shall have exactly the same IE type to represent a list.

Assigning the same IE type to grouped IEs which don't have the same content is not recommended, even if these grouped IEs are in different message levels.

#### 7.1.3.4	Information Element Type

An IE in a message or Grouped IE is identified by its IE Type and described by a specific row in the corresponding tables in clause 7.

If several IEs with the same Type are included in a message or Grouped IE, they represent a list for the corresponding IE name.

An IE Type value uniquely identifies a specific IE.

One IE type value is specified for Vendor Specific IEs.

## 7.2	Message Types

The message types are defined in Table 7.2-1.

# Table 7.2-1: Message Types

| Message Type value (Decimal) | Message |
| --- | --- |
| 0 | Reserved |
| 1 | Get Request |
| 2 | Get Response |
| 3 | Set Request |
| 4 | Set Response |
| 5 to 255 | For future use |

## 7.3	Messages

### 7.3.1	Get Request

#### 7.3.1.1	Message definition

The Get Request message is sent by the SMF/CUC to the AN-TL or CN-TL to retrieve ES parameters of the AN-TL or CN-TL, see Table 7.3.1.1-1.

Message type:	Get Request

Direction:	SMF/CUC to AN-TL, SMF/CUC to CN-TL

Table 7.3.1.1-1: Information Elements in Get Request

| Information elements | P | Condition / Comment | IE Type |
| --- | --- | --- | --- |
| Requested ES Parameters | M | This IE shall indicate the ES parameters requested to be retrieved from the AN-TL/CN-TL. | Requested ES Parameters |

### 7.3.2	Get Response

#### 7.3.2.1	Message definition

The Get Response message is sent by the AN-TL or CN-TL to the SMF/CUC to report the requested ES parameters of the AN-TL or CN-TL, see Table 7.3.2.1-1.

Message type:	Get Response

Direction:	AN-TL to SMF/CUC, CN-TL to SMF/CUC

Table 7.3.2.1-1: Information Elements in Get Response

| Information elements | P | Condition / Comment | IE Type |
| --- | --- | --- | --- |
| Cause | M | Indicates success or failure of the request, and in the latter case, the reason for the failure. | Cause |
| End Station Interface | C | This IE shall be included if the SMF/CUC has set the ES ITF bit to 1 in the Requested ES Parameters IE in the Get Request message. Several IEs with the same IE type may be present to report multiple ES interfaces. | End Station Interface |
| Interface Capabilities | C | This IE shall be included if the SMF/CUC has set the ITF CAP bit to 1 in the Requested ES Parameters IE in the Get Request message. | Interface Capabilities |

### 7.3.3	Set Request

#### 7.3.3.1	Message definition

The Set Request message is sent by the SMF/CUC to the AN-TL or CN-TL to configure ES parameters of the AN-TL or CN-TL, see Table 7.3.3.1-1.

Message type:	Set Request

Direction:	SMF/CUC to AN-TL, SMF/CUC to CN-TL

Table 7.3.3.1-1: Information Elements in Set Request

| Information elements | P | Condition / Comment | IE Type |
| --- | --- | --- | --- |
| Add TN Stream Configuration | C | This IE shall be present if it is requested to configure a new TN stream. See Table 7.3.3.1-2. Several IEs with the same IE type may be present to add multiple TN streams. | Add TN Stream Configuration |
| Modify TN Stream Configuration | C | This IE shall be present if it is requested to modify the configuration of an existing TN stream. See Table 7.3.3.1-3. Several IEs with the same IE type may be present to modify multiple TN streams. | Modify TN Stream Configuration |
| Delete TN Stream Configuration | C | This IE shall be present if it is requested to delete the configuration of an existing TN stream. See Table 7.3.3.1-4. Several IEs with the same IE type may be present to delete multiple TN streams. | Delete TN Stream Configuration |
| Parameters for Gate Control Information Calculation | O | This IE may be sent to a Talker. When present, it shall contain parameters that enable the AN-TL/CN-TL to calculate gate control information for the Interface/Port(s) associated with TN streams being added, modified or deleted. See Table 7.3.3.1-5. Several IEs with the same IE type may be present to provide parameters for gate control information calculation for multiple interfaces. | Parameters for Gate Control Information Calculation |

Table 7.3.3.1-2: Add TN Stream Configuration

| Octet 1 and 2 |  | Add TN Stream Configuration IE Type = 5 (decimal) |  |
| --- | --- | --- | --- |
| Octets 3 and 4 |  | Length = n |  |
| Information elements | P | Condition / Comment | IE Type |
| TN Stream ID | M | This IE shall contain a TN stream ID uniquely identifying the stream's configuration at the ES. | TN Stream ID |
| TN Stream Identification with mask-and-match | C | This IE shall be present if packets belonging to the TN stream are identified using mask-and-match information. (NOTE) | Mask-and-match information |
| TN Stream Identification with Data Frame Specification | C | This IE shall be present if packets belonging to the TN stream are identified using a Data Frame Specification. See Table 7.3.3.1-5. (NOTE) | Data Frame Specification |
| Interface configuration | M | Interface configuration of the ES for the TN stream, indicating an interface/port with which the TN stream is to be associated. It may also indicate a destination MAC address, PCP and/or VLAN ID, if stream transformation is supported by the AN-TL/CN-TL and required to be performed at the AN-TL/CN-TL. | Interface configuration |
| NOTE: Either the TN Stream Identification with mask-and-match IE or the TN Stream Identification with Data Frame Specification shall be present. |  |  |  |

Table 7.3.3.1-3: Modify TN Stream Configuration

| Octet 1 and 2 |  | Modify TN Stream Configuration IE Type = 6 (decimal) |  |
| --- | --- | --- | --- |
| Octets 3 and 4 |  | Length = n |  |
| Information elements | P | Condition / Comment | IE Type |
| TN Stream ID | M | This IE shall contain the TN stream ID of the stream configuration to be modified. | TN Stream ID |
| Interface configuration | C | This IE shall be present if a different port (Interface Name) needs to be used for the TN stream. When present, this IE shall be set as during the addition of the TN stream configuration but with the new port (Interface Name). If this IE is received, the AN-TL/CN-TL shall replace any earlier received Interface Configuration with the new information received in this IE. | Interface configuration |

Table 7.3.3.1-4: Delete TN Stream Configuration

| Octet 1 and 2 |  | Delete TN Stream Configuration IE Type = 7 (decimal) |  |
| --- | --- | --- | --- |
| Octets 3 and 4 |  | Length = n |  |
| Information elements | P | Condition / Comment | IE Type |
| TN Stream ID | M | This IE shall contain the TN stream ID of the stream's configuration to be deleted. | TN Stream ID |

Table 7.3.3.1-5: Data Frame Specification

| Octet 1 and 2 |  | Data Frame Specification IE Type = 8 (decimal) |  |
| --- | --- | --- | --- |
| Octets 3 and 4 |  | Length = n |  |
| Information elements | P | Condition / Comment | IE Type |
| Destination MAC Address | C | Destination MAC address. If absent, the destination MAC address shall be ignored for the purpose of Stream identification. | Destination MAC address |
| Source MAC Address | C | Source MAC address. If absent, the source MAC address shall be ignored for the purpose of Stream identification. | Source MAC address |
| VLAN Tag Info | C | Priority Code Point and/or VLAN ID used for stream identification | Vlan Tag Info |
| IPv4 tuple | C | IPv4 tuple for stream identification. | IPv4 tuple |
| IPv6 tuple | C | IPv6 tuple for stream identification. | IPv6 tuple |
| NOTE: At least one IE shall be present. |  |  |  |

Table 7.3.3.1-6: Parameters for Gate Control Information Calculation

| Octet 1 and 2 |  | Parameters for Gate Control Information Calculation IE Type = 9 (decimal) |  |
| --- | --- | --- | --- |
| Octets 3 and 4 |  | Length = n |  |
| Information elements | P | Condition / Comment | IE Type |
| Interface Name | C | When present, this IE shall indicate the Interface for which the TN stream parameters are provided. This IE shall be present if the ES supports multiple interfaces/ports. | Interface Name |
| Gate Control parameters | M | This IE shall contain Gate Control parameters of a TN stream mapped to the interface. Several IEs with the same IE type may be present to provide gate control parameters for multiple TN streams mapped to the interface. | Gate Control Parameters |

### 7.3.4	Set Response

#### 7.3.4.1	Message definition

The Set Response message is sent by the AN-TL or CN-TL to the SMF/CUC to report the outcomes of the ES parameters configuration request, see Table 7.3.4.1-1.

Message type:	Set Response

Direction:	AN-TL to SMF/CUC, CN-TL to SMF/CUC

Table 7.3.4.1-1: Information Elements in Set Response

| Information elements | P | Condition / Comment | IE Type |
| --- | --- | --- | --- |
| Cause | M | This IE shall indicate success, partial success or failure of the request, and in the latter case, the reason for the failure. | Cause |
| Partial Failure Information | C | This IE shall be present if the request was partially successful. When present, it shall indicate the TN streams for which the requested configuration failed to be applied and the reason of the failure. Several IEs with the same IE type may be present to report multiple streams. | Partial Failure Information |

Table 7.3.4.1-2: Partial Failure Information

| Octet 1 and 2 |  | Partial Failure Information IE Type = 20 (decimal) |  |
| --- | --- | --- | --- |
| Octets 3 and 4 |  | Length = n |  |
| Information elements | P | Condition / Comment | IE Type |
| TSN Stream ID | M | This IE shall indicate the TN Stream whose requested configuration failed to be applied. | TN Stream ID |
| Failure Cause | M | This IE shall indicate the reason why the requested stream configuration failed to be applied. | Cause |

## 7.4	Error Handling

### 7.4.1	Protocol Errors

A protocol error is defined as a message or an Information Element received from a peer entity with an unknown type, or if it is unexpected, or if it has an erroneous content.

The term silently discarded is used in the following clauses to mean that the receiving entity's implementation shall discard such a message without further processing or that the receiving entity discards such an IE and continues processing the message. The conditions for the receiving entity to silently discard an IE are specified in the subsequent clauses.

The handling of unknown, unexpected or erroneous messages and IEs shall provide for the forward compatibility of. Therefore, the sending entity shall be able to safely include in a message a new conditional-optional or an optional IE. Such an IE may also have a new type value. Any legacy receiving entity shall, however, silently discard such an IE and continue processing the message.

If a protocol error is detected by the receiving entity, it should log the event including the erroneous message and may include the error in a statistical counter.

For Response messages containing a rejection Cause value, see clause 7.1.3.2.

The receiving entity shall apply the error handling specified in the subsequent clauses.

If the received erroneous message is a reply to an outstanding message, the transaction layer shall stop retransmissions and notify the application layer of the error even if the reply is silently discarded.

### 7.4.2	Message of Invalid Length

If an entity receives a message, which is too short to contain the respective header, the PDU shall be silently discarded.

### 7.4.3	Unknown Message

If an entity receives a message with an unknown Message Type value, it shall silently discard the message.

### 7.4.4	Unexpected Message

If an entity receives an unexpected request message, for example a message for which the message is defined, but the direction is incorrect, then the entity shall silently discard the message and shall log an error.

If an entity receives an unexpected response message, for example a message for which there is no corresponding outstanding request, it shall discard the message and may log an error.

### 7.4.5	Missing Information Elements

An entity shall check if all mandatory IEs are present in the received Request message. If one or more mandatory information elements are missing in the received Request message, the entity should log the error and shall send a Response message with Cause IE value set to "Mandatory IE missing" with the type of the missing mandatory IE.

If an entity receives a Response message with Cause IE value set to "Mandatory IE missing", it shall notify its upper layer.

An entity shall check if all mandatory IEs are present in the received Response message without a rejection Cause value. If one or more mandatory information elements are missing, the entity shall notify the upper layer and should log the error.

An entity shall check if conditional information elements are present in the received Request message, if possible (i.e. if the receiving entity has sufficient information available to check if the respective conditions were met). If one or more conditional information elements are missing, an entity should log the error and shall send a Response message with Cause IE value set to "Conditional IE missing" together with the type of the missing conditional IE.

An entity shall check if conditional information elements are present in the received Response message without a rejection Cause value, if possible (i.e. if the receiving entity has sufficient information available to check if the respective conditions were met). If one or more conditional information elements are missing, an entity shall notify the upper layer and should log the error.

Absence of an optional information element shall not trigger any error handling.

### 7.4.6	Semantically incorrect Information Element

The receiver of a Request message including a mandatory or a verifiable conditional information element with a semantically invalid value shall discard the request, should log the error, and shall send a response with Cause IE value set to "Mandatory IE incorrect" together with a type and instance of the offending IE.

The receiver of a Response message including a mandatory or a verifiable conditional information element with a semantically invalid value shall notify the upper layer that a message with this sequence number has been received and should log the error.

If an entity receives an information element with a value which is shown as reserved, it shall treat that information element as invalid and should log the error. If the invalid IE is received in a Request, and it is a mandatory IE or a verifiable conditional IE, the entity shall send a response with Cause set to "Mandatory IE incorrect" together with a type and instance of the offending IE.

The use of reserved values invokes error handling; the use of spare values can be silently discarded and for IEs with spare values used, processing shall be continued ignoring the spare values.

The receiver of a signalling message including an optional information element with a value that is not in the range defined for this information element value shall discard this IE, but shall treat the rest of the message as if this IE was absent and continue processing. The receiver shall not check the content of an information element field that is defined as "spare".

All semantically incorrect optional information elements in a signalling message shall be treated as not present in the message.

### 7.4.7	Unknown or unexpected Information Element

The receiver of a message including an unexpected information element with a known Type value that is not defined for this message shall discard the IE and log an error. The receiver shall process the message.

NOTE:	An Information Element in an encoded message or grouped IE is identified by the IE Type.

### 7.4.8	Repeated Information Elements

An Information Element is repeated if there is more than one IE with the same IE Type in the scope of the message (or in the scope of the grouped IE). Such an IE is a member in a list.

If an information element is repeated in a signalling message in which repetition of the information element is not specified, only the contents of the information element appearing first shall be handled and all subsequent repetitions of the information element shall be ignored. When the number of repetitions of information elements is specified, only the contents of specified repeated information elements shall be handled and all subsequent repetitions of the information element shall be ignored.

# 8	Information elements coding

## 8.1	Information Elements Format

Figure 8.1-1 depicts the format of an Information Element.

|  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octets | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | 1 to 2 | Type = xxx (decimal) |  |  |  |  |  |  |  |  |
|  | 3 to 4 | Length = n |  |  |  |  |  |  |  |  |
|  | p to (p+1) | Enterprise ID |  |  |  |  |  |  |  |  |
|  | k to (n+4) | IE specific data or content of a grouped IE |  |  |  |  |  |  |  |  |

Figure 8.1-1: Information Element Format

NOTE 1:	If the Bit 8 of Octet 1 is not set, this indicates that the IE is defined by 3GPP and the Enterprise ID is absent. If Bit 8 of Octet 1 is set, this indicates that the IE is defined by a vendor and the Enterprise ID is present identified by the Enterprise ID.

An IE has the following mandatory fields:

-	Type: this field indicates the type of the Information Element. IE type values within the range of 0 to 32767 are reserved for IE defined by 3GPP and are listed in clause 8.2.1 IE type values within the range of 32768 to 65535 are used for vendor-specific IE and the value allocation is controlled by the vendor.

-	Length: this field contains the length of the IE excluding the first four octets, which are common for all IEs (Type and Length) and is denoted "n" in Figure 8.1-1 and in Figure 8.1-2. Bit 8 of the lowest numbered octet is the most significant bit and bit 1 of the highest numbered octet is the least significant bit.

An IE has the following optional fields:

-	Enterprise ID: if the IE type value is within the range of 32768 to 65535, this field shall contain the IANA-assigned "SMI Network Management Private Enterprise Codes" value of the vendor defining the IE. The Enterprise ID set to "10415" (IANA-assigned "SMI Network Management Private Enterprise Codes") shall not be used for the vendor specific IEs.

For illustration, Figure 8.1-2 depicts the format of an Information Element (IE) defined by 3GPP and is specified in this specification. For IE's defined by 3GPP, the IE type shall be within the range of 0 to 32767.

|  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octets | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | 1 to 2 | Type = xxx (decimal) |  |  |  |  |  |  |  |  |
|  | 3 to 4 | Length = n |  |  |  |  |  |  |  |  |
|  | 5 to (n+4) | IE specific data or content of a grouped IE |  |  |  |  |  |  |  |  |

Figure 8.1-2: 3GPP defined Information Element Format

NOTE 2:	Bit 8 of Octet 1 is not set. This indicates that the Information Element type value has been allocated by 3GPP.

For illustration, Figure 8.1-3 depicts the format of a vendor-specific Information Element, which content is not specified and the IE type value shall be within the range of 32768 to 65535.

|  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octets | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | 1 to 2 | Type = xxx (decimal) |  |  |  |  |  |  |  |  |
|  | 3 to 4 | Length = n |  |  |  |  |  |  |  |  |
|  | 5 to 6 | Enterprise ID |  |  |  |  |  |  |  |  |
|  | 7 to (n+4) | IE specific data or content of a grouped IE |  |  |  |  |  |  |  |  |

Figure 8.1-3: Vendor-Specific Information Element Format

NOTE 3:	Bit 8 of Octet 1 is set. This indicates that the IE type value has been allocated by the vendor identified by the Enterprise ID. The content of this IE is vendor specific and therefore out of scope of this specification.

## 8.2	Information Element Types

### 8.2.1	General

A message may contain several IEs. In order to have forward compatible type definitions for the Ies, all of them shall be TLV (Type, Length, Value) coded. IE type values are specified in the Table 8.2.1-1.

The 3rd column of this table specifies if the IE is either Extendable or has a variable length or a fixed length and a reference to the clause where the IE is specified:

-	Fixed Length: the IE has a fixed set of fields, and a fixed number of octets;

-	Variable Length: the IE has a fixed set of fields, and has a variable number of octets.
For example, the last octets may be numbered similar to "5 to (n+4)". In this example, if the value of the length field, n, is 0, then the last field is not present;

-	Extendable: the IE has a variable number of fields, and has a variable number of octets.
The last fields are typically specified with the statement: "These octet(s) is/are present only if explicitly specified". The legacy receiving entity shall ignore the unknown octets.

In order to improve the efficiency of troubleshooting, it is recommended that the IEs should be arranged in the signalling messages as well as in the grouped IEs, according to the order the IEs are listed in the message definition table or grouped IE definition table in clause 7. However the receiving entity shall be prepared to handle the messages with IEs in any order.

Within IEs, certain fields may be described as spare. These bits shall be transmitted with the value set to "0". To allow for future features, the receiver shall not evaluate these bits.

# Table 8.2.1-1: Information Element Types

| IE Type value (Decimal) | Information elements | Comment / Reference |
| --- | --- | --- |
| 0 | Reserved |  |
| 1 | Cause | Fixed Length / 8.2.2 |
| 2 | Requested ES Parameters | Extendable / 8.2.3 |
| 3 | End Station Interface | Extendable / 8.2.4 |
| 4 | Interface Capabilities | Extendable / 8.2.5 |
| 5 | Add TN Stream Configuration | Extendable / Table 7.3.1-2 |
| 6 | Modify TN Stream Configuration | Extendable / Table 7.3.1-3 |
| 7 | Delete TN Stream Configuration | Extendable / Table 7.3.1-4 |
| 8 | Data Frame Specification | Extendable / Table 7.3.1-5 |
| 9 | Parameters for Gate Control Information Calculation | Extendable / Table 7.3.1-6 |
| 10 | TN Stream ID | Fixed Length / 8.2.6 |
| 11 | Mask-and-match information | Extendable / 8.2.7 |
| 12 | Destination MAC address | Fixed Length / 8.2.8 |
| 13 | Source MAC address | Fixed Length / 8.2.9 |
| 14 | Vlan Tag Info | Fixed Length / 8.2.10 |
| 15 | IPv4 tuple | Extendable / 8.2.11 |
| 16 | IPv6 tuple | Extendable / 8.2.12 |
| 17 | Interface configuration | Extendable / 8.2.13 |
| 18 | Interface Name | Variable length / 8.2.14 |
| 19 | Gate Control Parameters | Extendable / 8.2.15 |
| 20 | Partial Failure Information | Extendable / Table 7.4.1-2 |
| 21 to 32767 | Spare. For future use. |  |
| 32768 to 65535 | Reserved for vendor specific IEs |  |

### 8.2.2	Cause

Cause IE is coded as depicted in Figure 8.2.2-1.

| . |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octets | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | 1 to 2 | Type = 1 (decimal) |  |  |  |  |  |  |  |  |
|  | 3 to 4 | Length = 1 |  |  |  |  |  |  |  |  |
|  | 5 | Cause value |  |  |  |  |  |  |  |  |

Figure 8.2.2-1: Cause

The Cause value shall be included in a response message. In a response message, the Cause value indicates the acceptance or the rejection of the corresponding request message. The Cause value indicates the explicit reason for the rejection.

Table 8.2.2-1: Cause values

| Message Type | Cause value (decimal) | Meaning | Description |
| --- | --- | --- | --- |
|  | 0 | Reserved. | Shall not be sent and if received the Cause shall be treated as an invalid IE |
| Acceptance in a response | 1 | Request accepted (success) | "Request accepted (success)" is returned when the request was accepted. |
|  | 2 | Request partially accepted | This cause shall be returned if the Set Request is partially accepted. |
|  | 3-63 | Spare | This value range shall be used by Cause values in an acceptance response message. See NOTE 1. |
| Rejection in a response | 64 | Request rejected (reason not specified) | This cause shall be returned to report an unspecified rejection cause |
|  | 65 | Mandatory IE missing | This cause shall be returned when the receiver detects that a mandatory IE is missing in a request message |
|  | 66 | Conditional IE missing | This cause shall be returned when the receiver detects that a Conditional IE is missing in a request message. |
|  | 67 | Invalid length | This cause shall be returned when the receiver detects that an IE with an invalid length in a request message |
|  | 68 | Mandatory IE incorrect | This cause shall be returned when the receiver detects that a Mandatory IE is incorrect in a request message, e.g. the Mandatory IE is malformed or it carries an invalid or unexpected value. |
|  | 69 to 255 | Spare for future use in a response message. See NOTE 2. | This value range shall be used by Cause values in a rejection response message. See NOTE 2. |
| NOTE 1: This value is or may be used in future version of the specification. If the receiver cannot comprehend the value, it shall be interpreted as an unspecified acceptance cause. Unspecified/unrecognized acceptance cause shall be treated in the same ways as the cause value 1 " Request accepted (success)". NOTE 2: This value is or may be used in a future version of the specification. If the receiver cannot comprehend the value, it shall be interpreted as an unspecified rejection cause. Unspecified/unrecognized rejection cause shall be treated in the same ways as the cause value 64 "Request rejected (reason not specified)". |  |  |  |

### 8.2.3	Requested ES Parameters

The Requested ES Parameters IE type shall be encoded as shown in Figure 8.2.3-1. It indicates the ES parameters that are requested to be retrieved from the AN-TL or CN-TL.

|  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octets | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | 1 to 2 | Type = 2 (decimal) |  |  |  |  |  |  |  |  |
|  | 3 to 4 | Length = n |  |  |  |  |  |  |  |  |
|  | 5 | Spare | Spare | Spare | Spare | Spare | Spare | ITF CAP | ES ITF |  |
|  | 6 to (n+4) | These octet(s) is/are present only if explicitly specified |  |  |  |  |  |  |  |  |

Figure 8.2.3-1: Requested ES Parameters

Octet 5 shall be encoded as follows:

-	Bit 1 – ES ITF (End Station Interfaces): when set to "1", this indicates a request to retrieve the ES interfaces.

-	Bit 2 – ITF CAP (Interface capabilities): when set to "1", this indicates a a request to retrieve the Interface capabilities.

-	Bit 3 to 8 Spare, for future use and set to 0.

### 8.2.4	End Station Interface

The End Station Interface IE type shall be encoded as shown in Figure 8.2.4-1. It conveys the description of one ES interface.

|  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octets | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | 1 to 2 | Type = 3 (decimal) |  |  |  |  |  |  |  |  |
|  | 3 to 4 | Length = n |  |  |  |  |  |  |  |  |
|  | 5 to 10 | MAC address value |  |  |  |  |  |  |  |  |
|  | 11 | Length of Interface Name |  |  |  |  |  |  |  |  |
|  | m to o | Interface Name value |  |  |  |  |  |  |  |  |
|  | p to (n+4) | These octet(s) is/are present only if explicitly specified |  |  |  |  |  |  |  |  |

Figure 8.2.4-1: End Station Interface

Octets 5 to 10 shall contain the value of the MAC address of the ES interface as specified in IEEE Std 802.1Q [4] Table 46-3.

Octet 11 shall contain the length of the Interface Name value field in octets. It shall be set to zero if no Interface Name value field is provided.

Octets m to o shall contain the value of the Interface Name of the ES interface as specified in IEEE Std 802.1Q [4] Table 46-3.

### 8.2.5	Interface Capabilities

The Interface Capabilities IE type shall be encoded as shown in Figure 8.2.5-1. It indicates the ES interface capabilities of the AN-TL or CN-TL to the SMF/CUC as defined in 3GPP TS 23.501 [2] table M.2-1.

|  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octets | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | 1 to 2 | Type = 4 (decimal) |  |  |  |  |  |  |  |  |
|  | 3 to 4 | Length = n |  |  |  |  |  |  |  |  |
|  | 5 | Spare | Spare | Spare | Spare | Spare | Spare | BUF CAP | VLAN TAG |  |
|  | m to (m+1) | Buffer Capability value |  |  |  |  |  |  |  |  |
|  | o to (n+4) | These octet(s) is/are present only if explicitly specified |  |  |  |  |  |  |  |  |

Figure 8.2.5-1: Interface Capabilities

Octet 5 shall be encoded as follows:

-	Bit 1 – VLAN TAG (VLAN Tag Capability): when set to "1", this indicates that the interface supports VLAN Tag capability as defined in IEEE Std 802.1Q [4] clause 46.2.3.7.1.

-	Bit 2 – BUF CAP (Buffer capabilities): when set to "1", this indicates that the Buffer Capability value field is provided.

-	Bit 3 to 8 Spare, for future use and set to 0.

Octet m to (m+1) shall be present if the BUF CAP flag in octet 5 is set to "1". When present, they encode, for a Talker, the maximum possible buffer duration in ms for a packet with the maximum size of an Ethernet packet (1522 Bytes).

### 8.2.6	TN Stream ID

The TN Stream ID IE type shall be encoded as shown in Figure 8.2.6-1.

|  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octets | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | 1 to 2 | Type = 10 (decimal) |  |  |  |  |  |  |  |  |
|  | 3 to 4 | Length = 8 |  |  |  |  |  |  |  |  |
|  | 5 to 10 | MAC address |  |  |  |  |  |  |  |  |
|  | 11 to 12 | Unique ID |  |  |  |  |  |  |  |  |

Figure 8.2.6-1: TN Stream ID

The MAC address shall be encoded as defined in clause 46.2.3.1.1 of IEEE Std 802.1Q [4].

The Unique ID shall be encoded as defined in clause 46.2.3.1.2 of IEEE Std 802.1Q [4].

### 8.2.7	Mask-and-match information

The Mask-and-match information IE type shall be encoded as shown in Figure 8.2.7-1.

|  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octets | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | 1 to 2 | Type = 11 (decimal) |  |  |  |  |  |  |  |  |
|  | 3 to 4 | Length = n |  |  |  |  |  |  |  |  |
|  | 5 to 6 | tsnCpeMmIdMsduMaskLength |  |  |  |  |  |  |  |  |
|  | 7 to m | tsnCpeMmIdMsduMask |  |  |  |  |  |  |  |  |
|  | (m+1) to p | tsnCpeMmIdMsduMatch |  |  |  |  |  |  |  |  |
|  | p+1 to (n+4) | These octet(s) is/are present only if explicitly specified |  |  |  |  |  |  |  |  |

Figure 8.2.7-1: Mask-and-match-information

Octets 5 to 6 shall encode the tsnCpeMmIdMsduMaskLength, i.e. the length, in octets, of tsnCpeMmIdMsduMask and and tsnCpeMmIdMsduMatch,as defined in clause 9.1.6.5 of IEEE Std  802.1CBdb [7].

Octets 7 to m shall encode the tsnCpeMmIdMsduMask, i.e. a mask of tsnCpeMmIdMsduMaskLength octets, as defined in clause 9.1.6.6 of IEEE Std  802.1CBdb [7].

Octets (m+1) to p shall encode the tsnCpeMmIdMsduMatch, i.e. the value of the mask to be matched for the stream identification, of tsnCpeMmIdMsduMaskLength octets, as defined in clause 9.1.6.7 of IEEE Std  802.1CBdb [7].

### 8.2.8	Destination MAC address

The Destination MAC address IE type shall be encoded as shown in Figure 8.2.8-1.

|  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octets | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | 1 to 2 | Type = 12 (decimal) |  |  |  |  |  |  |  |  |
|  | 3 to 4 | Length = 6 |  |  |  |  |  |  |  |  |
|  | 5 to 10 | MAC address value |  |  |  |  |  |  |  |  |

Figure 8.2.8-1: Destination MAC address

### 8.2.9	Source MAC address

The Source MAC address IE type shall be encoded as shown in Figure 8.2.9-1.

|  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octets | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | 1 to 2 | Type = 13 (decimal) |  |  |  |  |  |  |  |  |
|  | 3 to 4 | Length = 6 |  |  |  |  |  |  |  |  |
|  | 5 to 10 | MAC address value |  |  |  |  |  |  |  |  |

Figure 8.2.9-1: Source MAC address

### 8.2.10	VLAN Tag Info

The VLAN Tag Info IE type shall be encoded as shown in Figure 8.2.10-1.

|  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octets | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | 1 to 2 | Type = 14 (decimal) |  |  |  |  |  |  |  |  |
|  | 3 to 4 | Length = 2 |  |  |  |  |  |  |  |  |
|  | 5 | PCP value |  |  | DEI Flag | VLAN ID value |  |  |  |  |
|  | 6 | VLAN ID value |  |  |  |  |  |  |  |  |

Figure 8.2.10-1: VLAN Tag Info

The PCP value in octet 5 shall encode the Priority Code Point value (value range is 0 to 7 inclusive) as defined in clause 46.2.3.4.2 of IEEE Std 802.1Q [4].

The DEI (Drop Eligible Indicator) flag in octet 5 is not relevant from the perspective of a TSN Talker/Listener. It shall be ignored by the receiver.

The VLAN ID value field in octets 5 to 6 shall contain the VlanId value (value range is 0 to 4095 inclusive) as defined in clause 46.2.3.4.2 of IEEE Std 802.1Q [4]. Octet 5 / Bit 4 shall be the most significant bit of the VLAN ID value and Octet 6 / Bit 1 shall be the least significant bit.

### 8.2.11	IPv4 tuple

The IPv4 tuple IE type shall be encoded as shown in Figure 8.2.11-1. It encodes an IPv4 tuple as defined in clause 46.2.3.4.3 of IEEE Std 802.1Q [4].

|  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octets | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | 1 to 2 | Type = 15 (decimal) |  |  |  |  |  |  |  |  |
|  | 3 to 4 | Length = n |  |  |  |  |  |  |  |  |
|  | 5 | Spare | Spare | DPN | SPN | Protoc | DSCP | DIPA | SIPA |  |
|  | m to (m+3) | Source IPv4 address |  |  |  |  |  |  |  |  |
|  | o to (o+3) | Destination IPv4 address |  |  |  |  |  |  |  |  |
|  | p | Spare |  | DSCP value |  |  |  |  |  |  |
|  | q to (q+1) | Protocol value |  |  |  |  |  |  |  |  |
|  | r to (r+1) | Source Port Number |  |  |  |  |  |  |  |  |
|  | s to (s+1) | Destination Port Number |  |  |  |  |  |  |  |  |
|  | t to (n+4) | These octet(s) is/are present only if explicitly specified |  |  |  |  |  |  |  |  |

Figure 8.2.11-1: IPv4 tuple

Octet 5 shall be encoded as follows:

-	Bit 1 – SIPA (Source IP Address): when set to "1", this indicates that the Source IPv4 address field is present.

-	Bit 2 – DIPA (Destination IP Address): when set to "1", this indicates that the Destination IPv4 address field is present.

-	Bit 3 – DSCP: when set to "1", this indicates that the DSCP value field is present.

-	Bit 4 – Protoc (Protocol): when set to "1", this indicates that the Protocol value field is present.

-	Bit 5 – SPN (Source Port Number): when set to "1", this indicates that the Source Port Number field is present.

-	Bit 6 – DPN (Destination Port Number): when set to "1", this indicates that the Destination Port Number field is present.

-	Bit 7 to 8 Spare, for future use and set to 0.

When present, source IPv4 address, destination IPv4 address, Protocol, Source Port Number and Destination Port Number shall be encoded as defined in clause 46.2.3.4.3 of IEEE Std 802.1Q [4].

The DSCP value shall be encoded as the DSCP in TOS (IPv4) or TC (IPv6) information (see IETF RFC 2474 [8]).

### 8.2.12	IPv6 tuple

The IPv6 tuple IE type shall be encoded as shown in Figure 8.2.12-1. It encodes an IPv6 tuple as defined in clause 46.2.3.4.4 of IEEE Std 802.1Q [4].

|  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octets | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | 1 to 2 | Type = 16 (decimal) |  |  |  |  |  |  |  |  |
|  | 3 to 4 | Length = n |  |  |  |  |  |  |  |  |
|  | 5 | Spare | Spare | DPN | SPN | Protoc | DSCP | DIPA | SIPA |  |
|  | m to (m+15) | Source IPv6 address |  |  |  |  |  |  |  |  |
|  | o to (o+15) | Destination IPv6 address |  |  |  |  |  |  |  |  |
|  | p | Spare |  | DSCP value |  |  |  |  |  |  |
|  | q to (q+1) | Protocol value |  |  |  |  |  |  |  |  |
|  | r to (r+1) | Source Port Number |  |  |  |  |  |  |  |  |
|  | s to (s+1) | Destination Port Number |  |  |  |  |  |  |  |  |
|  | t to (n+4) | These octet(s) is/are present only if explicitly specified |  |  |  |  |  |  |  |  |

Figure 8.2.12-1: IPv6 tuple

Octet 5 shall be encoded as follows:

-	Bit 1 – SIPA (Source IP Address): when set to "1", this indicates that the Source IPv6 address field is present.

-	Bit 2 – DIPA (Destination IP Address): when set to "1", this indicates that the Destination IPv6 address field is present.

-	Bit 3 – DSCP: when set to "1", this indicates that the DSCP value field is present.

-	Bit 4 – Protoc (Protocol): when set to "1", this indicates that the Protocol value field is present.

-	Bit 5 – SPN (Source Port Number): when set to "1", this indicates that the Source Port Number field is present.

-	Bit 6 – DPN (Destination Port Number): when set to "1", this indicates that the Destination Port Number field is present.

-	Bit 7 to 8 Spare, for future use and set to 0.

When present, source IPv6 address, destination IPv6 address, Protocol, Source Port Number and Destination Port Number shall be encoded as defined in clause 46.2.3.4.4 of IEEE Std 802.1Q [4].

The DSCP value shall be encoded as the DSCP in TOS (IPv4) or TC (IPv6) information (see IETF RFC 2474 [8]).

### 8.2.13	Interface configuration

The Interface configuration IE type shall be encoded as shown in Figure 8.2.13-1.

|  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octets | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | 1 to 2 | Type = 17 (decimal) |  |  |  |  |  |  |  |  |
|  | 3 to 4 | Length = n |  |  |  |  |  |  |  |  |
|  | 5 | Spare |  |  |  | VLAN | PCP | DMAC | ITFN |  |
|  | m | Length of Interface Name |  |  |  |  |  |  |  |  |
|  | (m+1) to o | Interface Name value |  |  |  |  |  |  |  |  |
|  | p to (p+5) | Destination MAC address |  |  |  |  |  |  |  |  |
|  | q | Spare |  |  |  |  | PCP |  |  |  |
|  | r | Spare |  |  |  | VLAN ID value |  |  |  |  |
|  | r+1 | VLAN ID value |  |  |  |  |  |  |  |  |
|  | s to (n+4) | These octet(s) is/are present only if explicitly specified |  |  |  |  |  |  |  |  |

Figure 8.2.13-1: Interface configuration

Octet 5 shall be encoded as follows:

-	Bit 1 – ITFN (Interface Name): when set to "1", this indicates that the Length of Interface Name field and the Interface Name value field are present.

-	Bit 2 – DMAC (Destination MAC Address): when set to "1", this indicates that the Destination MAC address field is present.

-	Bit 3 – PCP: when set to "1", this indicates that octet q and the PCP value field are present.

-	Bit 4 – VLAN: when set to "1", this indicates that octets r and r+1 and the VLAN ID value field are present.

-	Bit 5 to 8 Spare, for future use and set to 0.

When present, the Length of Interface Name in octet m shall contains the length of the Interface Name value part in octets.

When present, the Interface Name value in octet (m+1) to o shall contain the value of the Interface Name of the ES interface as specified in IEEE Std 802.1Q [4] Table 46-3.

When present, the Destination MAC address field in octet p to (p+5) shall contain the destination MAC address, as defined in clause 46.2.3.4.1 of IEEE Std 802.1Q [4].

When octet q is present, the PCP value shall contain the Priority Code Point value (value range is 0 to 7 inclusive) as defined in clause 46.2.3.4.2 of IEEE Std 802.1Q [4].

When octets r to (r+1) are present, the VLAN ID value shall encode the VlanId value as defined in clause 46.2.3.4.2 of IEEE Std 802.1Q [4]. Octet r / Bit 4 shall be the most significant bit of the VLAN ID value and Octet (r+1) / Bit 1 shall be the least significant bit.

Editor's note: In Table 46-3 of IEEE Std 802.1Q [4], the InterfaceID contains the ES MacAddress and InterfaceName. Table M.2-1 of TS 23.501 defines only the InterfaceName for the Interface Configuration (Set Request). It is FFS whether this correct, i.e. whether the ES MacAddress should be added to Figure 8.2.13-1 and also to the Interface Configuration (Set Request) in Table M.2-1 of TS 23.501.

### 8.2.14	Interface Name

The Interface Name IE type shall be encoded as shown in Figure 8.2.14-1.

|  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octets | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | 1 to 2 | Type = 18 (decimal) |  |  |  |  |  |  |  |  |
|  | 3 to 4 | Length = n |  |  |  |  |  |  |  |  |
|  | 5 to n | Interface Name value |  |  |  |  |  |  |  |  |

Figure 8.2.14-1: Interface Name

The Interface Name value shall contain the value of the Interface Name of the ES interface as specified in IEEE Std 802.1Q [4] Table 46-3.

### 8.2.15	Gate Control Parameters

The Gate Control Parameters IE type shall be encoded as shown in Figure 8.2.15-1.

|  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octets | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | 1 to 2 | Type = 19 (decimal) |  |  |  |  |  |  |  |  |
|  | 3 to 4 | Length = n |  |  |  |  |  |  |  |  |
|  | 5 | Spare |  |  |  |  | TAO | MFS | INT |  |
|  | m to (m+3) | Interval numerator |  |  |  |  |  |  |  |  |
|  | o to (o+3) | Interval denominator |  |  |  |  |  |  |  |  |
|  | p to (p+1) | Max Frame Size |  |  |  |  |  |  |  |  |
|  | q to (q+3) | Time Aware Offset |  |  |  |  |  |  |  |  |
|  | r to (n+4) | These octet(s) is/are present only if explicitly specified |  |  |  |  |  |  |  |  |

Figure 8.2.15-1: Interface Name

Octet 5 shall be encoded as follows:

-	Bit 1 – INT (Interval): when set to "1", this indicates that the Interval numerator and Interval denominator fields are present.

-	Bit 2 – MFS (Max Frame Size): when set to "1", this indicates that the Max Frame Size field is present.

-	Bit 3 – TAO: when set to "1", this indicates that the Time Aware Offset field is present.

-	Bit 4 to 8 Spare, for future use and set to 0.

When present, the Interval numerator and Interval denominator fields shall contain the interval numerator and denominator as defined in Table 46-8 of IEEE Std 802.1Q [4].

When present, the Max Frame Size field shall contain the MaxFrameSize as defined in Table 46-8 of IEEE Std 802.1Q [4].

When present, the Time Aware Offset field shall contain the TimeAwareOffset as defined in clause 46.2.5.3.5 of IEEE Std 802.1Q [4].

Editor's note: A Set Request can include parameters to enable the Talker to calculate Gate Control Information, comprising the Interval, Max Frame Size and/or Time Aware Offset parameters of all the TN streams of all the PDU sessions mapped to a specific Interface/Port. Table M.2-1 of TS 23.501 does not require though the SMF/CUC to provide information that defines which TN stream, addressed in the Set-Request, is associated with a particular instance of the parameters (Interval, Max Frame Size, Time Aware Offset). This is also important when a single set request is used to add, modify, and delete multiple streams of a same PDU session and some requested stream configurations failed, because only such TN stream identification allows to identify which parameters to calculate the Gate Control information needs to be considered. It needs to be checked whether this is intentional to not include information like the TN StreamID to identify for each TN stream the particular instance of the parameters (Interval, Max Frame Size, Time Aware Offset).

# 9	Timers

Timers supported by the protocol.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2023-06 | CT4#116 | C4-232444 |  |  |  | Initial draft | 0.1.0 |
| 2023-09 | CT4#117 | C4-233809 |  |  |  | Implementation of agreed pCRs: C4-233162, C4-233163, C4-233164, C4-233165, C4-233166, C4-233167, C4-233168, C4-233169 | 0.2.0 |