3GPP TS 36.133 V14.13.0 (2019-09)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Evolved Universal Terrestrial Radio Access (E-UTRA);
Requirements for support of radio resource management

(Release 14)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

# Contents

Foreword	78

1	Scope	79

2	References	79

3	Definitions, symbols and abbreviations	80

3.1	Definitions	80

3.2	Symbols	82

3.3	Abbreviations	83

3.4	Test tolerances	85

3.5	Additional notation	85

3.5.1	Groups of bands	85

3.6	General	87

3.6.1	Applicability of requirements in this specification version	87

4	E-UTRAN RRC_IDLE state mobility	93

4.1	Cell Selection	93

4.2	Cell Re-selection	93

4.2.1	Introduction	93

4.2.2	Requirements	93

4.2.2.1	Measurement and evaluation of serving cell	94

4.2.2.2	Void	95

4.2.2.3	Measurements of intra-frequency E-UTRAN cells	95

4.2.2.4	Measurements of inter-frequency E-UTRAN cells	97

4.2.2.5	Measurements of inter-RAT cells	99

4.2.2.5.1	Measurements of UTRAN FDD cells	99

4.2.2.5.2	Measurements of UTRAN TDD cells	101

4.2.2.5.3	Measurements of GSM cells	102

4.2.2.5.4	Measurements of HRPD cells	103

4.2.2.5.5	Measurements of cdma2000 1X	104

4.2.2.6	Evaluation of cell re-selection criteria	105

4.2.2.7	Maximum interruption in paging reception	106

4.2.2.8	void	106

4.2.2.9	UE measurement capability	106

4.2.2.9a	UE measurement capability (Increased UE carrier monitoring)	107

4.2.2.10	Reselection to CSG cells	107

4.2.2.10.1	Reselection from a non CSG to an inter-frequency CSG cell	107

4.2.2.10.2	Reselection from a non CSG to an inter-RAT UTRAN FDD CSG cell	108

4.2.2.11	Void	109

4.2.2.12	Void	109

4.2.2.13	Void	109

4.3	Minimization of Drive Tests (MDT)	109

4.3.1	Introduction	110

4.3.2	Measurements	110

4.3.2.1	Requirements	110

4.3.3	Relative Time Stamp Accuracy	110

4.3.3.1	Requirements	110

4.3.4	Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	110

4.3.4.1	 Requirements	110

4.3.5	Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	111

4.3.5.1	Requirements for timeSinceFailure	111

4.4	MBSFN Measurements	111

4.4.1	Introduction	111

4.4.2	MBSFN RSRP measurements	111

4.4.3	MBSFN RSRQ measurements	111

4.4.4	MCH BLER measurements	112

4.5	Proximity-based Services	112

4.5.1	Introduction	112

4.5.2	Requirements	112

4.5.2.1	Interruptions with ProSe Direct Discovery	112

4.5.2.2	Interruptions with ProSe Direct Communication	112

4.5.2.3	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	112

4.5.2.4	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	113

4.6	Cell Selection and Re-selection Requirements for UE category NB1	113

4.6.1	Cell Selection	113

4.6.2	Cell Re-selection	113

4.6.2.1	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage	114

4.6.2.2	Measurements of intra-frequency NB-IoT cells for UE category NB1 in normal coverage	114

4.6.2.3	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in enhanced coverage	116

4.6.2.4	Measurements of intra-frequency NB-IoT cells for UE category NB1 in enhanced coverage	116

4.6.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage	118

4.6.2.6	Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage	119

4.6.2.7	Maximum interruption in paging reception in normal coverage	120

4.6.2.7A	Maximum interruption in paging reception in enhanced coverage	120

4.6.2.8	UE measurement capability	121

4.7	Cell Selection and Re-selection Requirements for UE category M1	121

4.7.1	Cell Selection	121

4.7.2	Cell Re-selection	121

4.7.2.1	Cell Re-selection requirements for UE category M1 in normal coverage	121

4.7.2.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	121

4.7.2.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	121

4.7.2.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	123

4.7.2.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	124

4.7.2.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	124

4.7.2.2	Cell Re-selection requirements for UE category M1 in enhanced coverage	125

4.7.2.2.1	Measurement and evaluation of serving cell for UE category M1 in enhanced coverage	125

4.7.2.2.2	Measurements of intra-frequency cells for UE category M1 in enhanced coverage	126

4.7.2.2.3	Measurements of inter-frequency cells for UE category M1 in enhanced coverage	127

4.7.2.2.4	Maximum allowed layers for multiple monitoring for UE category M1 in enhanced coverage	129

4.7.2.2.5	Maximum interruption in paging reception for Category M1 UEs in extended coverage	129

4.8	Idle State Positioning Measurement Requirements for UE category NB1	130

4.8.1	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for normal coverage	130

4.8.1.1	RSTD Measurement Reporting Delay	131

4.8.2	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	132

4.8.2.1	RSTD Measurement Reporting Delay	133

4.8.3	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for normal coverage	133

4.8.3.1	RSTD Measurement Reporting Delay	135

4.8.4	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	135

4.8.4.1	RSTD Measurement Reporting Delay	137

4.8.5	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	137

4.8.5.1	Measurement Reporting Delay	138

4.8.6	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	138

4.8.6.1	Measurement Reporting Delay	139

4.8.7	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	140

4.8.7.1	Measurement Reporting Delay	141

4.8.8	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	141

4.8.8.1	Measurement Reporting Delay	142

5	E-UTRAN RRC_CONNECTED state mobility	142

5.1	E-UTRAN Handover	143

5.1.1	Introduction	143

5.1.2	Requirements	143

5.1.2.1	E-UTRAN FDD – FDD	143

5.1.2.1.1	Handover delay	143

5.1.2.1.2	Interruption time	144

5.1.2.2	E-UTRAN FDD – TDD	145

5.1.2.2.1	(Void)	145

5.1.2.2.2	(Void)	145

5.1.2.3	E-UTRAN TDD – FDD	145

5.1.2.3.1	(Void)	145

5.1.2.3.2	(Void)	145

5.1.2.4	E-UTRAN TDD – TDD	145

5.1.2.4.1	Handover delay	145

5.1.2.4.2	Interruption time	146

5.1.2.5	E-UTRAN HD–FDD	147

5.1.2.5.1	Handover delay	147

5.1.2.5.2	Interruption time	147

5.2	Void	149

5.3	Handover to other RATs	149

5.3.1	E-UTRAN - UTRAN FDD Handover	149

5.3.1.1	Introduction	149

5.3.1.1.1	Handover delay	149

5.3.1.1.2	Interruption time	149

5.3.2	E-UTRAN - UTRAN TDD Handover	150

5.3.2.1	Introduction	150

5.3.2.2	Requirements	150

5.3.2.2.1	Handover delay	150

5.3.2.2.2	Interruption time	150

5.3.3	E-UTRAN - GSM Handover	151

5.3.3.1	Introduction	151

5.3.3.2	Requirements	151

5.3.3.2.1	Handover delay	151

5.3.3.2.2	Interruption time	151

5.4	Handover to Non-3GPP RATs	151

5.4.1	E-UTRAN – HRPD Handover	151

5.4.1.1	Introduction	151

5.4.1.1.1	Handover delay	152

5.4.1.1.2	Interruption time	152

5.4.2	E-UTRAN – cdma2000 1X Handover	152

5.4.2.1	Introduction	152

5.4.2.1.1	Handover delay	152

5.4.2.1.2	Interruption time	152

5.5	E-UTRAN Handover for Cat-M1 UEs	153

5.5.1	Introduction	153

5.5.2	Requirements in CEModeA	153

5.5.2.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	153

5.5.2.1.1	Handover delay	153

5.5.2.1.2	Interruption time	153

5.5.2.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	154

5.5.2.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	154

5.5.2.3.1	Void	154

5.5.2.3.2	Void	154

5.5.3	Requirements in CEModeB	154

5.5.3.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	154

5.5.3.1.1	Handover delay	154

5.5.3.1.2	Interruption time	155

5.5.3.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	155

5.5.3.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	155

5.6	Void	155

6	RRC Connection Mobility Control	155

6.1	RRC Re-establishment	155

6.1.1	Introduction	156

6.1.2	Requirements	156

6.1.2.1	UE Re-establishment delay requirement	156

6.2	Random Access	156

6.2.1	Introduction	156

6.2.2	Requirements	156

6.2.2.1	Contention based random access	157

6.2.2.1.1	Correct behaviour when receiving Random Access Response reception	157

6.2.2.1.2	Correct behaviour when not receiving Random Access Response reception	157

6.2.2.1.3	Correct behaviour when receiving a NACK on msg3	157

6.2.2.1.4	Void	157

6.2.2.1.5	Correct behaviour when receiving a message over Temporary C-RNTI	157

6.2.2.1.6	Correct behaviour when contention Resolution timer expires	157

6.2.2.2	Non-Contention based random access	157

6.2.2.2.1	Correct behaviour when receiving Random Access Response	157

6.2.2.2.2	Correct behaviour when not receiving Random Access Response	157

6.2.3	Requirements for Cat-M1 UEs	158

6.3	RRC Connection Release with Redirection	158

6.3.1	Introduction	158

6.3.2	Requirements	158

6.3.2.1	RRC connection release with redirection to UTRAN FDD	158

6.3.2.2	RRC connection release with redirection to GERAN	158

6.3.2.3	RRC connection release with redirection to UTRAN TDD	159

6.4	CSG Proximity Indication for E-UTRAN and UTRAN	159

6.4.1	Introduction	159

6.4.2	Requirements	159

6.5	RRC Re-establishment for NB-IoT UEs	160

6.5.1	Introduction	160

6.5.2	Requirements	160

6.5.2.1	UE Re-establishment delay requirement in normal coverage	160

6.5.2.2	UE Re-establishment delay requirement in enhanced coverage	161

6.6	Random Access for UE category NB1	161

6.6.1	Introduction	161

6.6.2	Requirements	161

6.6.2.1	Correct behaviour when receiving Random Access Response reception	161

6.6.2.2	Correct behaviour when not receiving Random Access Response reception	162

6.6.2.3	Correct behaviour when receiving a NACK on msg3	162

6.6.2.4	Correct behaviour when receiving a message over Temporary C-RNTI	162

6.6.2.5	Correct behaviour when contention Resolution timer expires	162

6.6.2.6	MSG3-based channel quality report for UE Category NB1	162

6.6.3	Requirements for NPRACH configuration	162

6.7	RRC Re-establishment for Cat-M1 UEs	163

6.7.1	Introduction	163

6.7.2	Requirements	163

6.7.2.1	UE Re-establishment delay requirement for CEModeA	163

6.7.2.2	UE Re-establishment delay requirement for CEModeB	164

6.8	RRC Connection Release with Redirection for Cat-M1 UEs	164

6.8.1	Introduction	164

6.8.2	Requirements	164

6.8.2.1	RRC connection release with redirection to E-UTRAN with CE Mode A	164

6.9	RRC Connection Redirection to Non-anchor Carrier in NB-IoT	165

6.9.1	Introduction	165

6.9.2	Requirements	165

7	Timing and signalling characteristics	166

7.1	UE transmit timing	166

7.1.1	Introduction	166

7.1.2	Requirements	166

7.2	UE timer accuracy	167

7.2.1	Introduction	167

7.2.2	Requirements	167

7.3	Timing Advance	168

7.3.1	Introduction	168

7.3.2	Requirements	168

7.3.2.1	Timing Advance adjustment delay	168

7.3.2.2	Timing Advance adjustment accuracy	168

7.4	Cell phase synchronization accuracy (TDD)	168

7.4.1	Definition	168

7.4.2	Minimum requirements	168

7.5	Synchronization Requirements for E-UTRAN to 1xRTT and HRPD Handovers	169

7.5.1	Introduction	169

7.5.2	eNodeB Synchronization Requirements	169

7.5.2.1	Synchronized E-UTRAN	169

7.5.2.2	Non-Synchronized E-UTRAN	169

7.6	Radio Link Monitoring	169

7.6.1	Introduction	169

7.6.2	Requirements	171

7.6.2.1	Minimum requirement when no DRX is used	171

7.6.2.2	Minimum requirement when DRX is used	171

7.6.2.3	Minimum requirement at transitions	173

7.6.2.4	Minimum requirement during SI Acquisition with autonomous gaps	173

7.6.2.5	Minimum requirement under IDC Interference	173

7.7	SCell Activation and Deactivation Delay for E-UTRA Carrier Aggregation	173

7.7.1	Introduction	173

7.7.2	SCell Activation Delay Requirement for Deactivated SCell	173

7.7.3	SCell Deactivation Delay Requirement for Activated SCell	174

7.7.4	SCell Activation Delay Requirement for Deactivated SCell with Muliple Downlnk SCells	174

7.7.5	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	175

7.7.6	SCell Activation Delay Requirement for Deactivated PUCCH SCell	176

7.7.7	SCell Activation Delay Requirement for Deactivated PUCCH SCell with Multiple SCells	176

7.7.8	SCell Deactivation Delay Requirement for Activated PUCCH SCell	177

7.7.9	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple SCells	177

7.7.10	SCell Activation Delay Requirement for Deactivated SCell under Frame Structure 3	177

7.7.11	SCell Deactivation Delay Requirement for Activated SCell under Frame Structure 3	178

7.7.12	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells under Frame Structure 3	179

7.7.13	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells under Frame Structure 3	179

7.8	Interruptions with Carrier Aggregation	179

7.8.1	Introduction	179

7.8.2	Requirements	180

7.8.2.1	Interruptions at SCell addition/release for intra-band CA	180

7.8.2.2	Interruptions at SCell addition/release for inter-band CA	180

7.8.2.3	Interruptions at SCell activation/deactivation for intra-band CA	180

7.8.2.4	Interruptions at SCell activation/deactivation for inter-band CA	180

7.8.2.5	Interruptions during measurements on SCC for intra-band CA	180

7.8.2.6	Interruptions during measurements on SCC for inter-band CA	181

7.8.2.7	Interruptions at SCell addition/release with multiple downlink SCells	181

7.8.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	181

7.8.2.9	Interruptions during measurements on SCC with multiple downlink SCells	181

7.8.2.10	Interruptions at overlapping addition/release/activation/deactivation of SCells	183

7.8.2.11	Interruptions during RSSI measurements on one SCC under Frame Structure 3	183

7.8.2.12	Interruptions during RSSI measurements on multiple SCCs under Frame Structure 3	183

7.8.2.13	Interruptions at SRS carrier based switching	184

7.9	Maximum Transmission Timing Difference in Carrier Aggregation	184

7.9.1	Introduction	184

7.9.2	Minimum Requirements for Interband Carrier Aggregation	184

7.9.3	Minimum Requirements for Intraband non-contiguous Carrier Aggregation	185

7.9.4	Minimum Requirements for Inter-Band Carrier Aggregation under Frame Structure 3	185

7.10	Interruptions with RSTD Measurements with Carrier Aggregation	185

7.10.1	Introduction	185

7.10.2	Requirements	185

7.10.2.1	Interruptions during RSTD measurements on SCC for intra-band CA with one downlink SCell	186

7.10.2.2	Interruptions during RSTD measurements on SCC for inter-band CA with one downlink SCell	186

7.10.2.3	Interruptions during RSTD measurements on SCC with multiple downlink SCells	186

7.10.2.4	Interruptions at overlapping RSTD and inter-frequency measurements	187

7.11	Radio Link Monitoring for UE Category 0	187

7.11.1	Introduction	187

7.11.2	Requirements for FD-FDD and TDD	188

7.11.2.1	Minimum requirement when no DRX is used	188

7.11.2.2	Minimum requirement when DRX is used	189

7.11.2.3	Minimum requirement at transitions	189

7.11.3	Requirements for HD-FDD	190

7.11.3.1	Minimum requirement when no DRX is used	190

7.11.3.2	Minimum requirement when DRX is used	190

7.11.3.3	Minimum requirement at transitions	191

7.12	Interruptions with Dual Connectivity	191

7.12.1	Introduction	191

7.12.2	Requirements	191

7.12.2.1	Interruptions at PSCell addition/release	191

7.12.2.2	Interruptions at transitions between active and non-active during DRX	191

7.12.2.3	Interruptions at transitions from non-DRX to DRX	192

7.12.2.4	Interruptions at SCell addition/release	192

7.12.2.5	Interruptions at SCell activation/deactivation	192

7.12.2.6	Interruptions during measurements on SCC	193

7.12.2.7	Interruptions at SRS carrier based switching	193

7.13	Cell phase synchronization accuracy (Synchronized mode of dual connectivity)	194

7.13.1	Definition	194

7.13.2	Minimum requirements	194

7.14	PSCell Addition and Release Delay for E-UTRA Dual Connectivity	194

7.14.1	Introduction	194

7.14.2	PSCell Addition Delay Requirement	194

7.14.3	PSCell Release Delay Requirement	195

7.15	Maximum Receive Timing Difference in Dual Connectivity	195

7.15.1	Introduction	195

7.15.2	Minimum Requirements for Inter-band Dual Connectivity	195

7.16	Proximity-based Services	195

7.16.1	Introduction	195

7.16.2	Requirements	195

7.16.2.1	ProSe UE transmission timing	195

7.16.2.1.1	Serving cell or PCell as timing reference	196

7.16.2.1.2	SCell or non-serving cell as timing reference	196

7.16.3	Interruptions with ProSe	196

7.16.3.1	Interruptions at ProSe Direct Discovery configuration	196

7.16.3.2	Interruptions at ProSe Direct Communication configuration	196

7.16.3.3	Interruptions during ProSe Direct Discovery	197

7.16.3.4	Interruptions during ProSe Direct Discovery with discovery gaps	197

7.16.3.5	Interruptions during ProSe Direct Communication	197

7.16.4	Cell reselection for ProSe Direct Discovery on non-serving frequency	198

7.16.4.1	Measurement and evaluation of selected cell	198

7.16.4.2	Measurement of intra-frequency E-UTRAN cells	198

7.16.5	Selection / Reselection of ProSe relay UE	199

7.16.6	ProSe operation under deactivated SCell	199

7.17	Maximum Transmission Timing Difference in Dual Connectivity	199

7.17.1	Introduction	199

7.17.2	Minimum Requirements for maximum transmission timing difference Inter-band Dual Connectivity	200

7.18.1	Introduction	200

7.18.2	SCell Activation Delay Requirement for Deactivated SCell	200

7.18.3	SCell Deactivation Delay Requirement for Activated SCell	200

7.19	Radio Link Monitoring for UE Category M1	200

7.19.1	Introduction	200

7.19.2	Requirements for FD-FDD and TDD CE mode A	200

7.19.2.1	Minimum requirement when no DRX is used	201

7.19.2.2	Minimum requirement when DRX is used	202

7.19.2.3	Minimum requirement at transitions	203

7.19.3	Requirements for HD-FDD with CE mode A	203

7.19.3.1	Minimum requirement when no DRX is used	204

7.19.3.2	Minimum requirement when DRX is used	204

7.19.3.3	Minimum requirement at transitions	205

7.19.4	Requirements for FD-FDD and TDD with CE mode B	205

7.19.4.1	Minimum requirement when no DRX is used	206

7.19.4.2	Minimum requirement when DRX is used	207

7.19.4.3	Minimum requirement at transitions	208

7.19.5	Requirements for HD-FDD with CE mode B	208

7.19.5.1	Minimum requirement when no DRX is used	209

7.19.5.2	Minimum requirement when DRX is used	209

7.19.5.3	Minimum requirement at transitions	210

7.20	UE transmit timing for NB-IoT	210

7.20.1	Introduction	210

7.20.2	Requirements	210

7.21	 UE timer accuracy for NB-IoT	211

7.21.1	Introduction	211

7.21.2	Requirements	211

7.22	Timing Advance for NB-IoT	211

7.22.1	Introduction	211

7.22.2	Requirements	211

7.22.2.1	Timing Advance adjustment delay	211

7.22.2.2	Timing Advance adjustment accuracy	212

7.23	Radio Link Monitoring for Category NB1 UE	212

7.23.1	Introduction	212

7.23.2	Requirements for HD-FDD Category NB1 UE	212

7.23.2.1	Minimum requirement when no DRX is used	212

7.23.2.2	Minimum requirement when DRX is used	213

7.23.2.3	Minimum requirement at transitions	213

7.24	UE transmit timing for Category M1	214

7.24.1	Introduction	214

7.24.2	Requirements	214

7.25	Cell phase synchronization accuracy for MBMS services (FDD)	215

7.25.1	Definition	215

7.25.2	Minimum requirements	215

7.26	UE transmit timing for Category M2	215

7.26.1	Introduction	215

7.26.2	Requirements	215

7.27	UE timer accuracy for category M1	216

7.27.1	Introduction	216

7.27.2	Requirements	216

7.28	Timing Advance for Category M1	216

7.28.1	Introduction	216

7.28.2	Requirements	216

7.29	Interruptions requirements with FeMBMS	216

7.29.1	Introduction	216

7.29.2	Requirements	216

7.30	Numerology switching delay requirements with FeMBMS	216

7.30.1	Introduction	216

7.30.2	Requirements	216

8	UE Measurements Procedures in RRC_CONNECTED State	217

8.1	General Measurement Requirements	217

8.1.1	Introduction	217

8.1.2	Requirements	217

8.1.2.1	UE measurement capability	217

8.1.2.1.1	Monitoring of multiple layers using gaps	221

8.1.2.1.1a	Monitoring of multiple layers using gaps (Increased UE carrier monitoring)	222

8.1.2.1.2	Network controlled small gap	223

8.1.2.2	E-UTRAN intra frequency measurements	225

8.1.2.2.1	E-UTRAN FDD intra frequency measurements	225

8.1.2.2.2	E-UTRAN TDD intra frequency measurements	229

8.1.2.2.3	E-UTRAN FDD intra frequency measurements with autonomous gaps	233

8.1.2.2.4	E-UTRAN TDD intra frequency measurements with autonomous gaps	234

8.1.2.2.5	E-UTRAN FDD intra-frequency measurements on carrier with FeMBMS/Unicast mixed cells	235

8.1.2.3	E-UTRAN inter frequency measurements	235

8.1.2.3.1	E-UTRAN FDD – FDD inter frequency measurements	236

8.1.2.3.2	E-UTRAN TDD – TDD inter frequency measurements	241

8.1.2.3.3	E-UTRAN TDD – FDD inter frequency measurements	248

8.1.2.3.4	E-UTRAN FDD – TDD inter frequency measurements	248

8.1.2.3.5	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	248

8.1.2.3.6	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	249

8.1.2.3.7	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	251

8.1.2.3.8	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	252

8.1.2.3.9	E-UTRAN FDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	253

8.1.2.3.10	E-UTRAN TDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	259

8.1.2.4	Inter RAT measurements	259

8.1.2.4.1	E-UTRAN FDD – UTRAN FDD measurements	259

8.1.2.4.2	E-UTRAN TDD – UTRAN FDD measurements	264

8.1.2.4.3	E-UTRAN TDD – UTRAN TDD measurements	264

8.1.2.4.4	E-UTRAN FDD – UTRAN TDD measurements	268

8.1.2.4.5	E-UTRAN FDD – GSM measurements	268

8.1.2.4.6	E-UTRAN TDD – GSM measurements	273

8.1.2.4.7	E-UTRAN FDD – UTRAN FDD measurements for SON	273

8.1.2.4.8	E-UTRAN TDD – UTRAN FDD measurements for SON	275

8.1.2.4.9	E-UTRAN FDD – cdma2000 1xRTT measurements	275

8.1.2.4.9.1A	E-UTRAN FDD – cdma2000 1xRTT measurements when no DRX is used	275

8.1.2.4.10	E-UTRAN TDD – cdma2000 1xRTT measurements	276

8.1.2.4.11	E-UTRAN FDD – HRPD measurements	276

8.1.2.4.12	E-UTRAN TDD – HRPD measurements	276

8.1.2.4.13	E-UTRAN TDD – UTRAN TDD measurements for SON	276

8.1.2.4.14	E-UTRAN FDD – UTRAN TDD measurements for SON	278

8.1.2.4.15	E-UTRAN FDD – cdma2000 1xRTT measurements for SON ANR	278

8.1.2.4.16	E-UTRAN TDD – cdma2000 1xRTT measurements for SON ANR	278

8.1.2.4.17	E-UTRAN FDD-UTRAN FDD measurements with autonomous gaps	278

8.1.2.4.18	E-UTRAN TDD-UTRAN FDD measurements with autonomous gaps	279

8.1.2.4.19	E-UTRAN FDD – WLAN measurements	279

8.1.2.4.20	E-UTRAN TDD – WLAN measurements	281

8.1.2.5	E-UTRAN OTDOA Intra-Frequency RSTD Measurements	281

8.1.2.5.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	281

8.1.2.5.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	283

8.1.2.5.3	E-UTRAN FDD Intra-Frequency OTDOA Measurements for UE Category 1bis	285

8.1.2.5.4	E-UTRAN TDD Intra-Frequency OTDOA Measurements for UE Category 1bis	286

8.1.2.6.5	(Void)	288

8.1.2.6.6	(Void)	288

8.1.2.6.7	(Void)	288

8.1.2.6.8	(Void)	288

8.1.2.6	E-UTRAN Inter-Frequency OTDOA Measurements	288

8.1.2.6.1	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements	289

8.1.2.6.2	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements	290

8.1.2.6.3	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements	292

8.1.2.6.4	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements	294

8.1.2.6.5	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	295

8.1.2.6.6	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	297

8.1.2.6.7	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	299

8.1.2.6.8	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	300

8.1.2.7	E-UTRAN E-CID Measurements	302

8.1.2.7.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	302

8.1.2.7.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	303

8.1.2.7.3	E-UTRAN FDD Intra-frequency E-CID RSRP and RSRQ Measurements	305

8.1.2.7.4	E-UTRAN TDD Intra-frequency E-CID RSRP and RSRQ Measurements	305

8.1.2.8	E-UTRAN intra-frequency measurements under time domain measurement resource restriction	306

8.1.2.8.1	E-UTRAN FDD intra-frequency measurements	306

8.1.2.8.2	E-UTRAN TDD intra-frequency measurements	309

8.1.2.8.3	E-UTRAN FDD intra-frequency measurements with CRS assistance information	312

8.1.2.8.4	E-UTRAN TDD intra-frequency measurements with CRS assistance infromation	315

8.1.2.9	E-UTRAN E-CID Measurements when Time Domain Measurement Resource Restriction Pattern is Configured	319

8.1.2.9.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	319

8.1.2.9.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	319

8.1.2.9.3	E-UTRAN FDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	319

8.1.2.9.4	E-UTRAN TDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	320

8.2	Capabilities for Support of Event Triggering and Reporting Criteria	320

8.2.1	Introduction	320

8.2.2	Requirements	321

8.3	Measurements for E-UTRA carrier aggregation	323

8.3.1	Introduction	323

8.3.2	Measurements of the primary component carrier	323

8.3.3	Measurements of a secondary component carrier	323

8.3.3.1	Measurements of a secondary component carrier with active SCell	323

8.3.3.2	Measurements of a secondary component carrier with deactivated SCell	324

8.3.3.2.1	E-UTRAN secondary component carrier measurements when no common DRX is used	324

8.3.3.2.2	E-UTRAN secondary component carrier measurements when common DRX is used	325

8.3.3.3	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and activated SCell	326

8.3.3.4	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and deactivated SCell	326

8.4	OTDOA RSTD Measurements for E-UTRAN carrier aggregation	327

8.4.1	Introduction	327

8.4.2	Measurements on the primary component carrier	327

8.4.3	Measurements on a secondary component carrier	328

8.4.4	Measurements on both primary component carrier and a secondary component carrier	328

8.4.5	Measurements on different secondary component carriers	329

8.5	Measurements for UE category 0	330

8.5.1	Introduction	330

8.5.2 	Requirements	331

8.5.2.1 	E-UTRAN intra frequency measurements	331

8.5.2.1.1 	E-UTRAN FDD intra frequency measurements	331

8.5.2.1.2 	E-UTRAN intra frequency measurements for HD-FDD	334

8.5.2.1.3	E-UTRAN TDD intra frequency measurements	336

8.5.2.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category 0	340

8.5.2.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category 0	341

8.5.2.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category 0	341

8.6	Discovery signal measurements	342

8.6.1	Introduction	342

8.6.2	Requirements for CRS based discovery signal measurements	342

8.6.2.1	E-UTRAN intra frequency measurements	342

8.6.2.1.1	E-UTRAN FDD intra frequency measurements	342

8.6.2.1.2	E-UTRAN TDD intra frequency measurements	345

8.6.2.2	E-UTRAN inter frequency measurements	347

8.6.2.2.1	E-UTRAN FDD – FDD inter-frequency measurements	348

8.6.2.2.2	E-UTRAN TDD – TDD inter frequency measurements	350

8.6.2.2.3	E-UTRAN TDD – FDD inter frequency measurements	353

8.6.2.2.4	E-UTRAN FDD – TDD inter frequency measurements	353

8.6.3	Requirements for CSI-RS based discovery signal measurements	353

8.6.3.1	E-UTRAN intra frequency measurements	353

8.6.3.1.1	E-UTRAN FDD intra frequency measurements	353

8.6.3.1.2	E-UTRAN TDD intra frequency measurements	356

8.6.3.2	E-UTRAN inter frequency measurements	358

8.6.3.2.1	E-UTRAN FDD – FDD inter frequency measurements	358

8.6.3.2.2	E-UTRAN TDD – TDD inter frequency measurements	361

8.6.3.2.3	E-UTRAN TDD – FDD inter frequency measurements	363

8.6.3.2.4	E-UTRAN FDD – TDD inter frequency measurements	364

8.7	Discovery signal measurements for E-UTRA carrier aggregation	364

8.7.1	Introduction	364

8.7.2	Requirements for CRS based discovery signal measurements for E-UTRA carrier aggregation	364

8.7.2.1	Measurements of the primary component carrier	364

8.7.2.2	Measurements of a secondary component carrier	364

8.7.2.3	Measurements of a secondary component carrier with active SCell	364

8.7.2.4	Measurements of a secondary component carrier with deactivated SCell	364

8.7.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	365

8.7.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	366

8.7.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	367

8.7.3.1	Measurements of the primary component carrier	367

8.7.3.2	Measurements of a secondary component carrier	367

8.7.3.3	Measurements of a secondary component carrier with active SCell	368

8.7.3.4	Measurements of a secondary component carrier with deactivated SCell	368

8.7.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	368

8.7.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	369

8.8	Measurements for E-UTRA dual connectivity	371

8.8.1	Introduction	371

8.8.2	Intra-frequency measurements requirements on PCell	371

8.8.3	Intra-frequency measurements requirements on PSCell	371

8.8.4	Inter-frequency and inter-RAT measurement requirements	371

8.8.5	Intra-frequency measurements with autonomous gaps	371

8.8.5.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	371

8.8.5.2	ECGI reporting delay	373

8.8.6	Inter-frequency measurements with autonomous gaps	373

8.8.6.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	373

8.8.6.2	ECGI reporting delay	374

8.8.7	SSTD Measurements	374

8.8.7.1	Introduction	374

8.8.7.2	SSTD Measurement requirements	374

8.8.7.3	SSTD Measurement Reporting Delay	374

8.8.8	Intra-frequency measurements requirements on SCell	375

8.9	MBSFN Measurements	375

8.9.1	Introduction	375

8.9.2	MBSFN RSRP Measurements	375

8.9.3	MBSFN RSRQ Measurements	375

8.9.4	MCH BLER Measurements	375

8.10	Proximity-based Services	376

8.10.1	Introduction	376

8.10.2	Requirements	376

8.10.2.1	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	376

8.10.2.2	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	376

8.11	Discovery Signal Measurements under Operation with Frame Structure 3	377

8.11.1	Introduction	377

8.11.2	CRS based discovery signal measurements	377

8.11.2.1	E-UTRAN intra-frequency measurements	377

8.11.2.1.1	Requirements	377

8.11.2.1.1.1	Requirements when no DRX is used	377

8.11.2.1.1.1.1	Measurement Reporting Requirements	379

8.11.2.1.1.2	Requirements when DRX is used	380

8.11.2.1.1.2.1	Measurement Reporting Requirements	382

8.11.2.2	E-UTRAN inter-frequency measurements	383

8.11.2.2.1	E-UTRAN FDD-FS3 inter-frequency measurements	383

8.11.2.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	387

8.11.3	CSI-RS based discovery signal measurements	387

8.11.3.1	E-UTRAN intra-frequency measurements	387

8.11.3.1.1	Requirements	387

8.11.3.1.1.1	Requirements when no DRX is used	387

8.11.3.1.1.1.1	Measurement Reporting Requirements	389

8.11.3.1.1.2	Requirements when DRX is used	389

8.11.3.1.1.2.1	Measurement Reporting Requirements	390

8.11.3.2	E-UTRAN inter-frequency measurements	391

8.11.3.2.1	E-UTRAN FDD – FS3 inter-frequency measurements	391

8.11.3.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	395

8.11.4	RSSI measurements	395

8.11.4.1	E-UTRAN intra-frequency measurements	395

8.11.4.2	E-UTRAN inter-frequency measurements	395

8.11.5	Channel occupancy measurements	395

8.11.5.1	E-UTRAN intra-frequency channel occupancy measurements	395

8.11.5.2	E-UTRAN inter-frequency channel occupancy measurements	396

8.12	Discovery Signal Measurements for E-UTRA Carrier Aggregation under Operation with Frame Structure 3	396

8.12.1	Introduction	396

8.12.2	CRS based discovery signal measurements for E-UTRA carrier aggregation	396

8.12.2.1	Introduction	396

8.12.2.2	Measurements of a secondary component carrier	396

8.12.2.3	Measurements of a secondary component carrier with active SCell	396

8.12.2.4	Measurements of a secondary component carrier with deactivated SCell	396

8.12.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	397

8.12.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	399

8.12.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	401

8.12.3.1	Introduction	401

8.12.3.2	Measurements of a secondary component carrier	401

8.12.3.3	Measurements of a secondary component carrier with active SCell	401

8.12.3.4	Measurements of a secondary component carrier with deactivated SCell	401

8.12.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	401

8.12.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	403

8.13	Measurements for UE Category M1	405

8.13.1	Introduction	405

8.13.2	Requirements for UE category M1 with CE mode A	405

8.13.2.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode A	406

8.13.2.1.1	E-UTRAN FDD intra frequency measurements	406

8.13.2.1.2	E-UTRAN intra frequency measurements for HD-FDD	410

8.13.2.1.3	E-UTRAN TDD intra frequency measurements	412

8.13.2.2	Void	415

8.13.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	415

8.13.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	415

8.13.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	418

8.13.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	420

8.13.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	421

8.13.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	421

8.13.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	423

8.13.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	425

8.13.2.5	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode A	426

8.13.2.5.1	Intra-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	426

8.13.2.5.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	426

8.13.2.5.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	426

8.13.2.5.4	Inter-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	427

8.13.2.5.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	427

8.13.2.5.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	428

8.13.2.5.7	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	428

8.13.2.5.8	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	429

8.13.2.5.9	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	430

8.13.2.6	E-UTRAN inter frequency measurements by UE category M1 with CE mode A	430

8.13.2.6.1	E-UTRAN FDD - FDD inter frequency measurements	430

8.13.2.6.2	E-UTRAN inter-frequency measurements for HD-FDD	434

8.13.2.6.3	E-UTRAN TDD inter frequency measurements	436

8.13.2.7	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode A	440

8.13.3	Requirements for UE category M1 with CE mode B	440

8.13.3.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode B	441

8.13.3.1.1	E-UTRAN FDD intra frequency measurements	441

8.13.3.1.2	E-UTRAN intra frequency measurements for HD-FDD	445

8.13.3.1.3	E-UTRAN TDD intra frequency measurements	448

8.13.3.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	453

8.13.3.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category M1 with CE mode B	454

8.13.3.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	454

8.13.3.2	Void	455

8.13.3.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	455

8.13.3.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	455

8.13.3.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	458

8.13.3.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	460

8.13.3.4	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode B	461

8.13.3.4.1	Intra-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	461

8.13.3.4.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	461

8.13.3.4.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	461

8.13.3.4.4	Inter-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	462

8.13.3.4.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	462

8.13.3.4.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	463

8.13.3.5	E-UTRAN inter frequency measurements by UE category M1 with CE Mode B	463

8.13.3.5.1	E-UTRAN FDD - FDD inter frequency measurements	463

8.13.3.5.2 	E-UTRAN inter-frequency measurements for HD-FDD	467

8.13.3.5.3	E-UTRAN TDD inter frequency measurements	469

8.13.3.6	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode B	474

8.13.3.7	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	475

8.13.3.7.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	475

8.13.3.7.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	477

8.13.3.7.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	479

8.14	Measurements for UE category NB1	480

8.14.1	Introduction	480

8.14.2	NB-IoT intra frequency measurements under normal coverage	480

8.14.2.1	NB-IoT intra frequency measurements when no DRX is used	480

8.14.2.2	NB-IoT intra frequency measurements when DRX is used	480

8.14.3	NB-IoT intra frequency measurements under enhanced coverage	480

8.14.3.1	NB-IoT intra frequency measurements when no DRX is used	480

8.14.3.2	NB-IoT intra frequency measurements when DRX is used	480

8.15	Void	481

8.16	Measurements for UE Category M2	481

8.16.1	Introduction	481

8.16.2	Requirements for UE category M2 with CE mode A	481

8.16.2.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	481

8.16.2.1.1	UE Rx-Tx Measurement Reporting Delay	481

8.16.2.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	482

8.16.2.2.1	UE Rx-Tx Measurement Reporting Delay	482

8.16.2.2a	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	483

8.16.2.2a.1	UE Rx-Tx Measurement Reporting Delay	483

8.16.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	483

8.16.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	483

8.16.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	486

8.16.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	488

8.16.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	489

8.16.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	489

8.16.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	491

8.16.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	493

8.16.3	Requirements for UE category M2 with CE mode B	494

8.16.3.1	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	494

8.16.3.1.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	494

8.16.3.1.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	496

8.16.3.1.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	499

8.16.3.2	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	499

8.16.3.2.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	499

8.16.3.2.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	501

8.16.3.2.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	504

8.17	Void	504

8.18	Measurements for non-BL/CE UE	504

8.18.1	Introduction	504

8.18.2	Requirements for non-BL/CE UE with CE Mode B	504

8.18.2.1	E-UTRAN intra frequency measurements	504

8.18.2.1.1	E-UTRAN FDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	504

8.18.2.1.2	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD non-BL/CE with CE Mode B	505

8.18.2.1.3	E-UTRAN TDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	506

9	Measurements performance requirements for UE	506

9.1	E-UTRAN measurements	507

9.1.1	Introduction	507

9.1.2	Intra-frequency RSRP Accuracy Requirements	507

9.1.2.1	Absolute RSRP Accuracy	507

9.1.2.2	Relative Accuracy of RSRP	508

9.1.2.3	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction	509

9.1.2.4	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction	509

9.1.2.5	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	510

9.1.2.6	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction with CRS assistance information	511

9.1.2.7	Absolute RSRP Accuracy for UE Category 1bis	512

9.1.2.8	Relative Accuracy of RSRP for UE Category 1bis	513

9.1.2A	Intra-frequency RSRP Accuracy Requirements in High Doppler Conditions	514

9.1.2A.1	Absolute RSRP Accuracy in high Doppler conditions	514

9.1.2A.2	Relative Accuracy of RSRP in high Doppler conditions	514

9.1.3	Inter-frequency RSRP Accuracy Requirements	515

9.1.3.1	Absolute RSRP Accuracy	515

9.1.3.2	Relative Accuracy of RSRP	516

9.1.3.3	Absolute RSRP Accuracy for UE Category 1bis	516

9.1.3.4	Relative Accuracy of RSRP for UE Category 1bis	517

9.1.3A	Inter-frequency RSRP Accuracy Requirements in High Doppler Conditions	518

9.1.3A.1	Absolute RSRP Accuracy in high Doppler conditions	518

9.1.3A.2	Relative Accuracy of RSRP in high Doppler conditions	518

9.1.4	RSRP Measurement Report Mapping	519

9.1.5	Intra-frequency RSRQ Accuracy Requirements	519

9.1.5.1	Absolute RSRQ Accuracy	519

9.1.5.2	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction	520

9.1.5.3	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	521

9.1.5.4	Absolute WB-RSRQ Accuracy	522

9.1.5.5	Absolute RSRQ Accuracy for UE Category 1bis	523

9.1.5A	Intra-frequency RSRQ Accuracy Requirements in High Doppler Conditions	524

9.1.5A.1	Absolute RSRQ Accuracy in high Doppler conditions	524

9.1.6	Inter-frequency RSRQ Accuracy Requirements	524

9.1.6.1	Absolute RSRQ Accuracy	524

9.1.6.2	Relative Accuracy of RSRQ	525

9.1.6.3	Absolute WB-RSRQ Accuracy	526

9.1.6.4	Relative WB-RSRQ Accuracy	526

9.1.6.5	Absolute RSRQ Accuracy for UE Category 1bis	527

9.1.6.6	Relative Accuracy of RSRQ for UE Category 1bis	528

9.1.6A	Inter-frequency RSRQ Accuracy Requirements in High Doppler Conditions	528

9.1.6A.1	Absolute RSRQ Accuracy in high Doppler conditions	528

9.1.6A.2	Relative Accuracy of RSRQ in high Doppler conditions	529

9.1.7	RSRQ Measurement Report Mapping	530

9.1.8	Power Headroom	530

9.1.8.1	Period	530

9.1.8.2	Reporting Delay	530

9.1.8.3	Void	531

9.1.8.4	Report Mapping	531

9.1.9	UE Rx – Tx time difference	531

9.1.9.1	Measurement Requirement	531

9.1.9.2	Measurement Report mapping	532

9.1.9.3	Measurement Requirement under Time Domain Measurement Resource Restriction	533

9.1.9.4	Measurement Requirement when Time Domain Measurement Resource Restriction Pattern is Configured with CRS Assistance Information	534

9.1.10	Reference Signal Time Difference (RSTD)	535

9.1.10.1	Intra-Frequency Accuracy Requirement	535

9.1.10.2	Inter-Frequency Accuracy Requirement	536

9.1.10.3	RSTD Measurement Report Mapping	538

9.1.10.4	Higher-Resolution RSTD Measurement Report Mapping	538

9.1.10.5	Intra-Frequency Accuracy Requirement for UE Category 1bis	539

9.1.10.6	Inter-Frequency Accuracy Requirement for UE Category 1bis	540

9.1.11	Carrier aggregation measurement accuracy	541

9.1.11.1	Primary component carrier accuracy requirement	542

9.1.11.2	Secondary component carrier accuracy requirement	542

9.1.11.3	Primary and secondary component carrier relative accuracy requirement	542

9.1.11.4	Secondary component carrier relative accuracy requirement	542

9.1.12	Reference Signal Time Difference (RSTD) Measurement Accuracy Requirements for Carrier Aggregation	542

9.1.13	Measurement accuracy for UE category 0	543

9.1.13.1	Intra-frequency Absolute RSRP Accuracy for UE category 0	543

9.1.13.2	Intra-frequency Relative Accuracy of RSRP for UE category 0	543

9.1.13.3	Intra-frequency Absolute RSRQ Accuracy for UE category 0	544

9.1.14	Accuracy requirements for Discovery Signal Measurements	545

9.1.14.1	Introduction	545

9.1.14.2	RSRP measurements in discovery signal occasions	545

9.1.14.3	CSI-RSRP measurements in discovery signal occasions	545

9.1.14.3.1	Intra-frequency CSI-RSRP measurements	545

9.1.14.3.1.1	Absolute CSI-RSRP measurement requirements	545

9.1.14.3.1.2	Relative CSI-RSRP measurement requirements	546

9.1.14.3.2	Inter-frequency CSI-RSRP measurements	547

9.1.14.3.2.1	Absolute CSI-RSRP measurement requirements	547

9.1.14.3.2.2	Relative CSI-RSRP measurement requirements	547

9.1.14.3.3	CSI-RSRP measurement report mapping	548

9.1.14.4	RSRQ measurements in discovery signal occasions	548

9.1.15	Discovery signal measurements accuracy for E-UTRAN carrier aggregation	548

9.1.15.1	Requirements for CRS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	549

9.1.15.1.1	Primary component carrier accuracy requirement	549

9.1.15.1.2	Secondary component carrier accuracy requirement	549

9.1.15.1.3	Primary and secondary component carrier relative accuracy requirement	549

9.1.15.1.4	Secondary component carrier relative accuracy requirement	549

9.1.15.2	Requirements for CSI-RS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	549

9.1.15.2.1	Primary component carrier accuracy requirement	549

9.1.15.2.2	Secondary component carrier accuracy requirement	549

9.1.15.2.3	Primary and secondary component carrier relative accuracy requirement	549

9.1.15.2.4	Secondary component carrier relative accuracy requirement	549

9.1.16	Accuracy requirements for RSRQ measurement on all OFDM symbols	550

9.1.17	RS-SINR Measurements	550

9.1.17.1	Measurement Report Mapping	550

9.1.17.2	Intra-frequency RS-SINR Measurement Accuracy Requirements	551

9.1.17.2.1	Absolute RS-SINR Measurement Accuracy Requirements	551

9.1.17.3	Inter-frequency RS-SINR Measurement Accuracy Requirements	551

9.1.17.3.1	Absolute RS-SINR Measurement Accuracy Requirements	551

9.1.17.3.2	Relative RS-SINR Measurement Accuracy Requirements	552

9.1.18	Accuracy Requirements for Measurements under Operation with Frame Structure 3	553

9.1.18.1	Introduction	553

9.1.18.2	RSRP measurements	553

9.1.18.2.1	RSRP measurement report mapping	553

9.1.18.2.2	Inter-frequency absolute RSRP measurement accuracy requirements	553

9.1.18.2.3	Inter-frequency relative RSRP measurement accuracy requirements	554

9.1.18.2.4	Intra-frequency absolute RSRP measurement accuracy requirements	554

9.1.18.2.5	Intra-frequency relative RSRP measurement accuracy requirements	555

9.1.18.3	RSRQ measurements	555

9.1.18.3.1	RSRQ measurement report mapping	555

9.1.18.3.2	Inter-frequency absolute RSRQ measurement accuracy requirements	555

9.1.18.3.3	Inter-frequency relative RSRQ measurement accuracy requirements	556

9.1.18.3.4	Intra-frequency absolute RSRQ measurement accuracy requirements	556

9.1.18.4	CSI-RSRP measurements	557

9.1.18.4.1	CSI-RSRP measurement report mapping	557

9.1.18.4.2	Inter-frequency absolute CSI-RSRP measurement accuracy requirements	557

9.1.18.4.3	Inter-frequency relative CSI-RSRP measurement accuracy requirements	557

9.1.18.4.4	Intra-frequency absolute CSI-RSRP measurement accuracy requirements	558

9.1.18.4.5	Intra-frequency relative CSI-RSRP measurement accuracy requirements	558

9.1.18.5	RSSI measurements	559

9.1.18.5.1	RSSI measurement report mapping	559

9.1.18.5.2	Intra-frequency absolute RSSI measurement accuracy requirements	559

9.1.18.5.3	Inter-frequency absolute RSSI measurement accuracy requirements	560

9.1.18.6	Channel occupancy measurements	560

9.1.18.6.1	Intra-frequency channel occupancy measurement accuracy requirements	560

9.1.18.6.2	Inter-frequency channel occupancy measurement accuracy requirements	560

9.1.19	Accuracy Requirements for Carrier Aggregation for Measurements under Operation with Frame Structure 3	561

9.1.19.1	Introduction	561

9.1.19.2	Accuracy requirements for measurements on SCC	561

9.1.19.3	Relative accuracy requirements for measurements on different SCCs	561

9.1.19.4	Relative accuracy requirements for measurements on SCC and PCC	561

9.1.20	SFN and Subframe Time Difference (SSTD)	562

9.1.20.1	SSTD Accuracy Requirement	562

9.1.20.2	SSTD Measurement Report Mapping	562

9.1.21	Measurement accuracy for UE category M1	563

9.1.21.1	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	563

9.1.21.2	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	564

9.1.21.3	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	565

9.1.21.4	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	566

9.1.21.5	RSRP Measurement Report Mapping	567

9.1.21.6	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode A	567

9.1.21.7	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode B	568

9.1.21.8	RSRQ Measurement Report Mapping	569

9.1.21.9	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	569

9.1.21.10	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	570

9.1.21.11	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	571

9.1.21.12	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	572

9.1.21.13 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode A	573

9.1.21.14	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode A	574

9.1.21.15 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode B	575

9.1.21.16	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode B	576

9.1.21.17	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	577

9.1.21.18	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	578

9.1.21.19	UE RX-TX time difference Accuracy Requirement for Cat-M1	579

9.1.21.20	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	580

9.1.21.21	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	582

9.1.22	Measurement accuracy for UE Category NB1	584

9.1.22.1	Intra-frequency Absolute NRSRP Accuracy for UE Category NB1	584

9.1.22.2	Void	584

9.1.22.3	Intra-frequency Absolute NRSRQ Accuracy for UE Category NB1 in Normal Mode	584

9.1.22.4	Void	585

9.1.22.5	Inter-frequency Absolute NRSRP Accuracy for UE Category NB1	585

9.1.22.6	Void	585

9.1.22.7	Inter-frequency Absolute NRSRQ Accuracy for UE Category NB1	585

9.1.22.8	Void	586

9.1.22.9	NRSRP Measurement Report Mapping	586

9.1.22.10	Intra-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	586

9.1.22.11	Inter-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	587

9.1.22.12	Intra-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	588

9.1.22.13	Inter-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	589

9.1.22.14	NRSRQ Measurement Report Mapping	590

9.1.22.15	MSG3-based Measurement Report Mapping for UE Category NB1	591

9.1.22.16	Downlink Channel Quality Measurement Accuracy for UE Category NB1	591

9.1.23	Power Headroom for UE Category NB1	592

9.1.23.1	Period	592

9.1.23.2	Reporting Delay	592

9.1.23.3	Report Mapping for UE Category NB1	592

9.1.23.3.1	Void	593

9.1.23.3.2	Void	593

9.1.23.4	Report Mapping for UE Category NB1 for UE Power Class 6	593

9.1.24	Void	593

9.1.25	Measurement accuracy for UE category M2	593

9.1.25.1	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	593

9.1.25.2	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	594

9.1.25.3	UE RX-TX time difference Accuracy Requirement for Cat-M2	595

9.1.25.4	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	596

9.1.25.5	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	598

9.1.26	Measurement Accuracy for non-BL CE UE	599

9.1.26.1	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	600

9.1.26.2	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	600

9.1.26.3	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	601

9.1.26.4	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	602

9.1.26.5	RSRP Measurement Report Mapping	603

9.1.26.6	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	603

9.1.26.7	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	603

9.1.26.8	RSRQ Measurement Report Mapping	604

9.1.26.9	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	604

9.1.26.10	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	605

9.1.26.11	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	606

9.1.26.12	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	607

9.1.26.13	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	608

9.1.26.14	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode A	608

9.1.26.15	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	608

9.1.26.16	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode B	609

9.2	UTRAN FDD Measurements	610

9.2.1	UTRAN FDD CPICH RSCP	611

9.2.2	Void	611

9.2.3	UTRAN FDD CPICH Ec/No	611

9.3	UTRAN TDD Measurements	612

9.3.1	UTRAN TDD P-CCPCH RSCP	612

9.3.2	Void	612

9.3.3	Void	612

9.4	GSM Measurements	612

9.4.1	GSM carrier RSSI	612

9.5	CDMA2000 1x RTT Measurements	613

9.5.1	CDMA2000 1x RTT Pilot Strength	613

9.6	PCMAX,c	613

9.6.1	Report Mapping	613

9.6.2 	Estimation Period	613

9.6.3	Reporting Delay	613

9.7	IEEE802.11 Measurements	613

9.7.1	WLAN RSSI	614

9.7.2	WLAN RSSI Measurement Report Mapping	614

9.8	MBSFN Measurements	614

9.8.1	Introduction	614

9.8.2	MBSFN RSRP	614

9.8.2.1	Absolute MBSFN RSRP measurement accuracy requirements	614

9.8.2.2	MBSFN RSRP measurement report mapping	615

9.8.2.3	MBSFN RSRP measurement report mapping for 7.5 kHz subcarrier spacing	615

9.8.2.4	MBSFN RSRP measurement report mapping for 1.25 kHz subcarrier spacing	616

9.8.3	MBSFN RSRQ	616

9.8.3.1	Absolute MBSFN RSRQ measurement accuracy requirements	616

9.8.3.2	MBSFN RSRQ measurement report mapping	616

9.8.3.3	MBSFN RSRQ measurement report mapping for 7.5 kHz subcarrier spacing	617

9.8.3.4	MBSFN RSRQ measurement report mapping for 1.25 kHz subcarrier spacing	617

9.8.4	MCH BLER	617

9.8.4.1	Measurement report mapping for MCH BLER	617

9.8.4.2	Measurement report mapping for MCH Block Number	618

9.9	ProSe Measurements	619

9.9.1	Introduction	619

9.9.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	619

9.9.2.1	Absolute S-RSRP Accuracy	619

9.9.2.2	Relative Accuracy of S-RSRP	620

9.9.3	Intra-Frequency SD-RSRP Measurement Accuracy Requirements	621

9.9.3.1	Absolute SD-RSRP Accuracy	621

9.9.3.2	Relative Accuracy of SD-RSRP	621

9.10	V2X Measurements	622

9.10.1	Introduction	622

9.10.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	622

9.10.2.1	Absolute S-RSRP Accuracy	622

9.10.2.2	Relative Accuracy of S-RSRP	623

9.10.3	PSSCH-RSRP Measurement Accuracy Requirements	623

9.10.3.1	Intra-frequency Absolute PSSCH-RSRP Accuracy	623

9.10.4	S-RSSI Measurement Accuracy Requirements	624

9.10.4.1	Intra-frequency absolute S-RSSI measurement accuracy requirements	624

9.10.4.2 Intra-frequency relative S-RSSI measurement accuracy requirements	624

10	Measurements Performance Requirements for E-UTRAN	625

10.1	Received Interference Power	625

10.1.1	Absolute accuracy requirement	625

10.1.2	Relative accuracy requirement	625

10.1.3	Received Interference Power measurement report mapping	625

10.2	Angle of Arrival (AOA)	626

10.2.1	Range/mapping	626

10.3	Timing Advance (TADV)	626

10.3.1	Report mapping	626

11	ProSe Requirements in Any Cell Selection state	627

11.1	Introduction	627

11.2	UE Transmit Timing for ProSe in Any Cell Selection State	627

11.2.1	Introduction	627

11.2.2	ProSe UE transmission timing	627

11.3	Initiation/Cease of SLSS Transmissions	627

11.3.1	Introduction	627

11.3.2	Requirements	627

11.4	Measurements for ProSe in Any Cell Selection State	628

11.4.1	Introduction	628

11.4.2	Requirements	628

11.4.2.1	E-UTRA FDD	628

11.4.2.2	E-UTRA TDD	628

11.5	Selection / Reselection of ProSe Synchronization Reference	629

11.5.1	Introduction	629

11.5.2	Selection/Reselection to intra-frequency SyncRef UE	629

11.5.2.1	Introduction	629

11.5.2.2	Requirements	629

11.6	Void	629

11.7	Selection / Reselection of ProSe relay UE	629

11.7.1	Introduction	629

11.7.2	Selection / Reselection of intra-frequency ProSe relay UE	630

12	V2V Sidelink Communication Requirements for V2V Operation on Dedicated V2V Carrier	630

12.1	Introduction	630

12.2	Transmit Timing	630

12.2.1	GNSS as timing reference	630

12.3	Interruption	631

12.4	Reliability of GNSS signal	631

13	V2X Requirements	631

13.1	Introduction	631

13.2	UE Transmit Timing	631

13.2.1	Introduction	631

13.2.2	GNSS as synchronization reference source	631

13.2.3	Serving cell/PCell as synchronization reference source	632

13.2.4	SyncRef UE as synchronization reference source	632

13.3	Initiation/Cease of SLSS Transmissions	632

13.3.1	Introduction	632

13.3.1.1	Initiation/Cease of SLSS transmissions with Serving cell / PCell as synchronization reference source	632

13.3.1.2	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	633

13.3.1.3	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	633

13.4	Selection / Reselection of V2X Synchronization Reference Source	633

13.5	Autonomous Resource Selection/Reselection measurements	634

13.5.1	Introduction	634

13.5.2	PSSCH-RSRP measurements	634

13.5.3	S-RSSI measurements	635

13.6	Congestion Control measurements	635

13.7	Interruption	635

13.7.1	Interruptions to WAN due to V2X Sidelink Communication	635

13.7.2	V2X Sidelink Communication Dropping due to synchronization reference source change	635

13.8	Reliability of GNSS signal	636

Annex A (normative):	Test Cases	637

A.1	Purpose of annex	637

A.2	Requirement classification for statistical testing	637

A.2.1	Types of requirements in TS 36.133	637

A.2.1.1	Time and delay requirements on UE higher layer actions	637

A.2.1.2	Measurements of power levels, relative powers and time	637

A.2.1.3	Implementation requirements	638

A.2.1.4	Physical layer timing requirements	638

A.3	RRM test configurations	639

A.3.1	Reference Measurement Channels	639

A.3.1.1	PDSCH	639

A.3.1.1.1	FDD	639

A.3.1.1.2	TDD	640

A.3.1.1.3	FDD for UE category 0	642

A.3.1.1.4	HD-FDD for UE category 0	643

A.3.1.1.5	TDD for UE category 0	644

A.3.1.1.6	Frame Structure 3	645

A.3.1.2	PCFICH/PDCCH/PHICH	646

A.3.1.2.1	FDD	646

A.3.1.2.2	TDD	646

A.3.1.2.3	HD-FDD for UE category 0	647

A.3.1.2.4	FS 3	647

A.3.1.3	MPDCCH Reference Channels for Cat-M1 UEs	647

A.3.1.3.1	FDD in CEModeA	648

A.3.1.3.2	HD-FDD in CEModeA	648

A.3.1.3.3	TDD in CEModeA	649

A.3.1.3.4	FDD in CEModeB	649

A.3.1.3.5	HD-FDD in CEModeB	650

A.3.1.3.6	TDD in CEModeB	650

A.3.1.4	PDSCH Reference Channel for Cat-M1 UEs	651

A.3.1.4.1	FDD in CEModeA	651

A.3.1.4.2	HD-FDD in CEModeA	652

A.3.1.4.3	TDD in CEModeA	653

A.3.1.4.4	FDD in CEModeB	654

A.3.1.4.5	 HD-FDD in CEModeB	655

A.3.1.4.6	TDD in CEModeB	655

A.3.1.5	NPDSCH Reference Channel for UE category NB1	656

A.3.1.5.1	HD-FDD in-band operation	656

A.3.1.5.2	Void	657

A.3.1.5.3	HD-FDD standalone operation	657

A.3.1.5.4	Void	658

A.3.1.5.5	HD-FDD guard band operation	658

A.3.1.5.6	Void	658

A.3.1.6	NPDCCH Reference Channel for UE category NB1	658

A.3.1.6.1	HD-FDD in-band operation	658

A.3.1.6.2	Void	659

A.3.1.6.3	HD-FDD standalone operation	659

A.3.1.6.4	Void	659

A.3.1.6.5	HD-FDD guard band operation	659

A.3.1.6.6	Void	660

A.3.2	OFDMA Channel Noise Generator (OCNG)	660

A.3.2.1	OCNG Patterns for FDD	660

A.3.2.1.1	OCNG FDD pattern 1: outer resource blocks allocation in 10 MHz	661

A.3.2.1.2	OCNG FDD pattern 2: full bandwidth allocation in 10 MHz	661

A.3.2.1.3	OCNG FDD pattern 3: outer resource blocks allocation in 1.4 MHz	662

A.3.2.1.4	OCNG FDD pattern 4: full bandwidth allocation in 1.4 MHz	663

A.3.2.1.5	OCNG FDD pattern 5: outer resource blocks allocation in 10 MHz (without MBSFN)	664

A.3.2.1.6	OCNG FDD pattern 6: full bandwidth allocation in 10 MHz (without MBSFN)	664

A.3.2.1.7	OCNG FDD pattern 7: full bandwidth allocation in 1.4 MHz (without MBSFN)	665

A.3.2.1.8	OCNG FDD pattern 8: outer resource blocks allocation in 10 MHz for MBSFN ABS	665

A.3.2.1.9	OCNG FDD pattern 9: full bandwidth allocation in 10 MHz for MBSFN ABS	666

A.3.2.1.10	OCNG FDD pattern 10: outer resource blocks allocation in 10 MHz with user data in every subframe (without MBSFN)	666

A.3.2.1.11	OCNG FDD pattern 11: outer resource blocks allocation in 20 MHz	667

A.3.2.1.12	OCNG FDD pattern 12: full bandwidth allocation in 20 MHz	668

A.3.2.1.13	OCNG FDD pattern 13: outer resource blocks allocation in 20 MHz (without MBSFN)	668

A.3.2.1.14	OCNG FDD pattern 14: full bandwidth allocation in 20 MHz (without MBSFN)	669

A.3.2.1.15	OCNG FDD pattern 15: outer resource blocks allocation in 5 MHz	669

A.3.2.1.16	OCNG FDD pattern 16: full bandwidth allocation in 5 MHz	670

A.3.2.1.17	OCNG FDD pattern 17: outer resource blocks allocation in 20 MHz with user data in every subframe (without MBSFN)	670

A.3.2.1.18	OCNG FDD pattern 18: outer resource blocks allocation in 5 MHz (without MBSFN)	671

A.3.2.1.19	OCNG FDD pattern 19: full bandwidth allocation in 5 MHz (without MBSFN)	671

A.3.2.1.20	OCNG FDD pattern 20: outer resource blocks allocation in 5 MHz with user data in every subframe (without MBSFN)	672

A.3.2.1.21	OCNG FDD pattern 21: Generic resource blocks allocation (without MBSFN)	672

A.3.2.1.22	OCNG FDD pattern 22: Generic resource blocks allocation in 5MHz (without MBSFN)	673

A.3.2.2	OCNG Patterns for TDD	673

A.3.2.2.1	OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	674

A.3.2.2.2	OCNG TDD pattern 2: full bandwidth allocation in 10 MHz	674

A.3.2.2.3	OCNG TDD pattern 3: outer resource blocks allocation in 1.4 MHz	675

A.3.2.2.4	OCNG TDD pattern 4: full bandwidth allocation in 1.4 MHz	675

A.3.2.2.5	OCNG TDD pattern 5: outer resource blocks allocation in 10 MHz for MBSFN ABS	676

A.3.2.2.6	OCNG TDD pattern 6: full bandwidth allocation in 10 MHz for MBSFN ABS	677

A.3.2.2.7	OCNG TDD pattern 7: outer resource blocks allocation in 20 MHz	678

A.3.2.2.8	OCNG TDD pattern 8: full bandwidth allocation in 20 MHz	679

A.3.2.2.9	OCNG TDD pattern 9: outer resource blocks allocation in 5 MHz	679

A.3.2.2.10	OCNG TDD pattern 10: full bandwidth allocation in 5 MHz	680

A.3.2.2.11	OCNG TDD pattern 11: Generic resource blocks allocation (without MBSFN)	681

A.3.2.3	OCNG Patterns for Narrowband IoT	681

A.3.2.3.1	Narrowband IoT OCNG FDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	683

A.3.2.3.2	Narrowband IoT OCNG FDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	684

A.3.2.3.3	Narrowband IoT OCNG FDD pattern 3: standalone NB-IoT	684

A.3.2.3.4	Narrowband IoT OCNG FDD pattern 4: In-band NB-IoT in 5 MHz EUTRAN cell	685

A.3.2.3.5	Narrowband IoT OCNG FDD pattern 5: guard band NB-IoT in 5 MHz EUTRAN cell	686

A.3.2.4	OCNG Patterns for V2X sidelink	687

A.3.2.4.1	V2X sidelink OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	688

A.3.2.4.2	V2X sidelink OCNG TDD pattern 2: outer resource blocks allocation in 10 MHz	688

A.3.3	Reference DRX Configurations	688

A.3.4	ABS Transmission Configurations	689

A.3.4.1	Non-MBSFN ABS Transmission Configurations	689

A.3.4.1.1	Non-MBSFN ABS Transmission, 1x2 antenna with PBCH	689

A.3.4.1.2	Non-MBSFN ABS Transmission, 2x2 antenna without PBCH	689

A.3.4.2	MBSFN ABS Transmission Configurations	690

A.3.4.2.1	MBSFN ABS Transmission, 1x2 antenna	690

A.3.4.2.2	MBSFN ABS Transmission, 2x2 antenna	691

A.3.5	Impact of Reference Sensitivity Degradation with Carrier Aggregation on Test Cases	691

A.3.5.1	Impact of Reference Sensitivity Degradation due to Insertion Loss	691

A.3.6	Carrier Aggregation Test Cases with Different Channel Bandwidth Combinations	692

A.3.6.1	Introduction	692

A.3.7	Test Cases with Different Channel Bandwidths	692

A.3.7.1	Introduction	692

A.3.7.2	Principle of testing	692

A.3.8	Antenna Configuration	692

A.3.8.1	Antenna connection for 4 Rx capable UEs	692

A.3.8.1.1 Introduction	692

A.3.8.1.2 Principle of testing	692

A.3.8.1.2.1 Single carrier tests	692

A.3.8.1.2.2	Carrier aggregation and Dual connectivity tests	693

A.3.8.1.2.3	Antenna connection for bands where 2RX is supported	694

A.3.8.1.2.4	Antenna connection for bands where 4RX is supported	694

A.3.9	Carrier Aggregation Test Cases with Different Duplex Modes	694

A.3.9.1	Introduction	694

A.3.9.2	Principle of testing	694

A.3.10	Carrier Aggregation Test Cases with Different CA Configurations	695

A.3.10.1	Introduction	695

A.3.10.2	Principle of testing	695

A.3.11	Test Cases for Synchronous and Asynchronous Dual Connectivity	695

A.3.11.1	Introduction	695

A.3.11.2	Principle of Testing	695

A.3.12	Proximity-based Services	695

A.3.12.1	Introduction	695

A.3.12.2	Reference DRX configurations for ProSe tests	695

A.3.12.3	Test Cases with Different Channel Bandwidths	696

A.3.12.3.1	Introduction	696

A.3.12.3.2	Principle of testing	696

A.3.12.4	Reference resource pool configurations for ProSe Direct Discovery	696

A.3.12.5	Reference resource pool configurations for ProSe Direct Communication	702

A.3.12.6	Reference Measurement Channels for ProSe Direct Discovery	704

A.3.12.6.1	FDD	704

A.3.12.7	Reference measurement channels for ProSe Direct Communication	704

A.3.12.7.1	FDD	704

A.3.12.8	ProSe Receive Traffic Generator	705

A.3.12.8.1	ProSe Direct Communication Receive Traffic Generator for FDD	705

A.3.12.8.2	ProSe Direct Discovery Receive Traffic Generator for FDD	705

A.3.13	Time Offset between Cells	706

A.3.13.1	Introduction	706

A.3.13.2	Definition	706

A.3.14	Carrier Aggregation under operation with Frame Structure 3 Test Cases with Different Duplex Modes	706

A.3.14.1	Introduction	706

A.3.14.2	Principle of testing	706

A.3.15	Dual connectivity test cases with different combination of duplex mode	706

A.3.15.1	Introduction	706

A.3.15.2	Principle of testing	706

A.3.16	Reference PRACH Configurations	707

A.3.17	Listen before talk model	707

A.3.17.1	Introduction	707

A.3.17.2	Definition	707

A.3.18	Reference NPRACH Configurations	708

A.3.19	Dual connectivity test cases with different bandwidth combinations	709

A.3.19.1	Introduction	709

A.3.19.2	Principle of testing	709

A.3.20	Category M1 UE Test Cases	709

A.3.20.1	Introduction	709

A.3.20.2	Principle of Cat-M1 UE Testing	709

A.3.21	V2V Sidelink Communication on Dedicated V2V Carrier	710

A.3.21.1	Introduction	710

A.3.21.2	Reference resource pool configurations for V2V Sidelink Communication	711

A.3.21.3	Reference measurement channels for V2V Sidelink Communication	712

A.3.22	Category 1bis UE Test Cases	713

A.3.22.1	Introduction	713

A.3.22.2	Principle of Category 1bis UE Testing	713

A.3.23	Category NB2 UE Test Cases	716

A.3.23.1	Introduction	716

A.3.23.2	Principle of Category NB2 UE Testing	716

A.3.24	V2X sidelink communication	717

A.3.24.1	Introduction	717

A.3.24.2	Reference resource pool configurations for V2X Sidelink Communication	718

A.3.24.3	Reference measurement channels for V2X Sidelink Communication	721

A.3.25	Category M2 UE Test Cases	722

A.3.25.1	Introduction	722

A.3.25.2	Principle of Cat-M2 UE Testing	722

A.4	E-UTRAN RRC_IDLE state	724

A.4.2	Cell Re-Selection	724

A.4.2.1	E-UTRAN FDD – FDD Intra frequency case	724

A.4.2.1.1	Test Purpose and Environment	724

A.4.2.1.2	Test Requirements	725

A.4.2.2	E-UTRAN TDD – TDD Intra frequency case	726

A.4.2.2.1	Test Purpose and Environment	726

A.4.2.2.2	Test Requirements	727

A.4.2.3	E-UTRAN FDD – FDD Inter frequency case	728

A.4.2.3.1	Test Purpose and Environment	728

A.4.2.3.2	Test Requirements	729

A.4.2.4	E-UTRAN FDD – TDD Inter frequency case	730

A.4.2.4.1	Test Purpose and Environment	730

A.4.2.4.2	Test Requirements	731

A.4.2.5	E-UTRAN TDD – FDD Inter frequency case	732

A.4.2.5.1	Test Purpose and Environment	732

A.4.2.5.2	Test Requirements	733

A.4.2.6	E-UTRAN TDD – TDD: Inter frequency case	734

A.4.2.6.1	Test Purpose and Environment	734

A.4.2.6.2	Test Requirements	735

A.4.2.7	E-UTRAN FDD – FDD Inter frequency case in the existence of non-allowed CSG cell	736

A.4.2.7.1	Test Purpose and Environment	736

A.4.2.7.2	Test Requirements	737

A.4.2.8	E-UTRAN TDD – TDD Inter frequency case in the existence of non-allowed CSG cell	738

A.4.2.8.1	Test Purpose and Environment	738

A.4.2.8.2	Test Requirements	739

A.4.2.9	E-UTRAN FDD – FDD Intra frequency case for 5MHz bandwidth	740

A.4.2.9.1	Test Purpose and Environment	740

A.4.2.9.2	Test Requirements	740

A.4.2.10	E-UTRAN FDD – FDD reselection using an increased number of carriers	740

A.4.2.10.1	Test Purpose and Environment	740

A.4.2.10.2	Test Requirements	744

A.4.2.11	E-UTRAN TDD – TDD reselection using an increased number of carriers	744

A.4.2.11.1	Test Purpose and Environment	744

A.4.2.11.2	Test Requirements	748

A.4.2.12	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage	748

A.4.2.12.1	Test Purpose and Environment	748

A.4.2.12.2	Test Requirements	750

A.4.2.13	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage	751

A.4.2.13.1	Test Purpose and Environment	751

A.4.2.13.2	Test Requirements	752

A.4.2.14	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	753

A.4.2.14.1	Test Purpose and Environment	753

A.4.2.14.2	Test Requirements	754

A.4.2.15 E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	755

A.4.2.15.1	Test Purpose and Environment	755

A.4.2.15.2	Test Requirements	756

A.4.2.16 E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	757

A.4.2.16.1	Test Purpose and Environment	757

A.4.2.16.2	Test Requirements	758

A.4.2.17 E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in enhanced coverage	759

A.4.2.17.1	Test Purpose and Environment	759

A.4.2.17.2	Test Requirements	760

A.4.2.18 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	761

A.4.2.18.1	Test Purpose and Environment	761

A.4.2.18.2	Test Requirements	763

A.4.2.19	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	764

A.4.2.19.1	Test Purpose and Environment	764

A.4.2.19.2	Test Requirements	766

A.4.2.20	E-UTRAN FDD – FDD Intra frequency case for UE Category 1bis	767

A.4.2.20.1	Test Purpose and Environment	767

A.4.2.20.2	Test Requirements	768

A.4.2.21	E-UTRAN TDD – TDD Intra frequency case for UE Category 1bis	769

A.4.2.21.1	Test Purpose and Environment	769

A.4.2.21.2	Test Requirements	770

A.4.2.22	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	771

A.4.2.22.1	Test Purpose and Environment	771

A.4.2.22.2	Test Requirements	772

A.4.2.23	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	773

A.4.2.23.1	Test Purpose and Environment	773

A.4.2.23.2	Test Requirements	774

A.4.2.24	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	775

A.4.2.24.1	Test Purpose and Environment	775

A.4.2.24.2	Test Requirements	778

A.4.2.25	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	779

A.4.2.25.1	Test Purpose and Environment	779

A.4.2.25.2	Test Requirements	780

A.4.2.26	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage	781

A.4.2.26.1	Test Purpose and Environment	781

A.4.2.26.2	Test Requirements	782

A.4.2.27	E-UTRAN TDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	783

A.4.2.27.1	Test Purpose and Environment	783

A.4.2.27.2	Test Requirements	784

A.4.2.28	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	785

A.4.2.28.1	Test Purpose and Environment	785

A.4.2.28.2	Test Requirements	786

A.4.2.29	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	787

A.4.2.29.1	Test Purpose and Environment	787

A.4.2.29.2	Test Requirements	788

A.4.2.30	E-UTRAN TDD Inter frequency case for Cat-M1 UE in enhanced coverage	789

A.4.2.30.1	Test Purpose and Environment	789

A.4.2.30.2	Test Requirements	790

A.4.2.31	E-UTRAN FDD – FDD Inter frequency case for UE Category 1bis	791

A.4.2.31.1	Test Purpose and Environment	791

A.4.2.31.2	Test Requirements	792

A.4.2.32	E-UTRAN FDD – TDD Inter frequency case for UE Category 1bis	793

A.4.2.32.1	Test Purpose and Environment	793

A.4.2.32.2	Test Requirements	794

A.4.2.33	E-UTRAN TDD – FDD Inter frequency case for UE Category 1bis	795

A.4.2.33.1	Test Purpose and Environment	795

A.4.2.33.2	Test Requirements	796

A.4.2.34	E-UTRAN TDD – TDD: Inter frequency case for UE Category 1bis	797

A.4.2.34.1	Test Purpose and Environment	797

A.4.2.34.2	Test Requirements	798

A.4.3	E-UTRAN to UTRAN Cell Re-Selection	799

A.4.3.1	E-UTRAN FDD – UTRAN FDD:	799

A.4.3.1.1	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of higher priority	799

A.4.3.1.1.1	Test Purpose and Environment	799

A.4.3.1.1.2	Test Requirements	801

A.4.3.1.2	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority	801

A.4.3.1.2.1	Test Purpose and Environment	801

A.4.3.1.2.2	Test Requirements	804

A.4.3.1.3	EUTRA FDD-UTRA FDD cell reselection in fading propagation conditions: UTRA FDD is of lower priority	804

A.4.3.1.3.1	Test Purpose and Environment	804

A.4.3.1.3.2	Test Requirements	807

A.4.3.1.4	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority for 5MHz bandwidth	807

A.4.3.1.4.1	Test Purpose and Environment	807

A.4.3.1.4.2	Test Requirements	808

A.4.3.1.5	Idle mode FDD to UTRA FDD interRAT reselection	808

A.4.3.1.5.1	Test Purpose and Environment	808

A.4.3.1.5.2	Test Requirements	811

A.4.3.2	E-UTRAN FDD – UTRAN TDD:	812

A.4.3.2.1	Test Purpose and Environment	812

A.4.3.2.1.1	Void	812

A.4.3.2.1.2	1.28Mcps TDD option	812

A.4.3.2.1.3	Void	814

A.4.3.2.2	Test Requirements	814

A.4.3.2.2.1	1.28Mcps TDD option	814

A.4.3.2A	E-UTRA FDD to UTRA TDD cell re-selection for IncMon	814

A.4.3.2A.1	Test Purpose and Environment	814

A.4.3.2A.2	Test Requirements	818

A.4.3.3	E-UTRAN TDD – UTRAN FDD:	818

A.4.3.3.1	Test Purpose and Environment	818

A.4.3.3.2	Test Requirements	821

A.4.3.3A	Idle mode TDD to UTRA FDD interRAT reselection	821

A.4.3.3A.1	Test Purpose and Environment	821

A.4.3.3A.2	Test Requirements	825

A.4.3.4	E-UTRAN TDD – UTRAN TDD:	826

A.4.3.4.1	E-UTRA to UTRA TDD cell re-selection: UTRA is of higher priority	826

A.4.3.4.1.1	Test Purpose and Environment	826

A.4.3.4.1.2	Test Requirements	828

A.4.3.4.2	E-UTRA to UTRA TDD cell re-selection: UTRA is of lower priority	828

A.4.3.4.2.1	Test Purpose and Environment	828

A.4.3.4.2.2	Test Requirements	830

A.4.3.4.3	EUTRA TDD-UTRA TDD cell reselection in fading propagation conditions: UTRA TDD is of lower priority	830

A.4.3.4.3.1	Test Purpose and Environment	830

A.4.3.4.3.2	Test Requirements	833

A.4.3.4.4	E-UTRA TDD to UTRA TDD cell re-selection for IncMon	833

A.4.3.4.4.1	Test Purpose and Environment	833

A.4.3.4.4.2	Test Requirements	837

A.4.4	E-UTRAN to GSM Cell Re-Selection	837

A.4.4.1	E-UTRAN FDD – GSM:	837

A.4.4.1.1	Test Purpose and Environment	837

A.4.4.1.2	Test Requirements	839

A.4.4.2	E-UTRAN TDD – GSM:	839

A.4.4.2.1	Test Purpose and Environment	839

A.4.4.2.2	Test Requirements	841

A.4.5	E-UTRAN to HRPD Cell Re-Selection	842

A.4.5.1	E-UTRAN FDD – HRPD	842

A.4.5.1.1	E-UTRAN FDD – HRPD Cell Reselection: HRPD is of Lower Priority	842

A.4.5.1.1.1	Test Purpose and Environment	842

A.4.5.1.1.2	Test Requirements	844

A.4.5.2	E-UTRAN TDD – HRPD	844

A.4.5.2.1	E-UTRAN TDD – HRPD Cell Reselection: HRPD is of Lower Priority	844

A.4.5.2.1.1	Test Purpose and Environment	844

A.4.5.2.1.2	Test Requirements	847

A.4.6	E-UTRAN to cdma2000 1X Cell Re-Selection	847

A.4.6.1	E-UTRAN FDD – cdma2000 1X	847

A.4.6.1.1	E-UTRAN FDD – cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	847

A.4.6.1.1.1	Test Purpose and Environment	847

A.4.6.1.1.2	Test Requirements	850

A.4.6.2	E-UTRAN TDD – cdma2000 1X	850

A.4.6.2.1	E-UTRAN TDD –cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	850

A.4.6.2.1.1	Test Purpose and Environment	850

A.4.6.2.1.2	Test Requirements	853

A.4.7	Idle State Positioning Measurement for UE category NB1	853

A.4.7.1	HD – FDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	853

A.4.7.1.1	Test Purpose and Environment	853

A.4.7.1.2	Test Requirements	857

A.4.7.2	HD – FDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	858

A.4.7.2.1	Test Purpose and Environment	858

A.4.7.4.2	Test Requirements	861

A.5	E-UTRAN RRC CONNECTED Mode Mobility	862

A.5.1	E-UTRAN Handover	862

A.5.1.1	E-UTRAN FDD - FDD Intra frequency handover	862

A.5.1.1.1	Test Purpose and Environment	862

A.5.1.1.2	Test Requirements	864

A.5.1.2	E-UTRAN TDD - TDD Intra frequency handover	864

A.5.1.2.1	Test Purpose and Environment	864

A.5.1.2.2	Test Requirements	866

A.5.1.3	E-UTRAN FDD – FDD Inter frequency handover	866

A.5.1.3.1	Test Purpose and Environment	866

A.5.1.3.2	Test Requirements	868

A.5.1.4	E-UTRAN TDD – TDD Inter frequency handover	868

A.5.1.4.1	Test Purpose and Environment	868

A.5.1.4.2	Test Requirements	870

A.5.1.5	E-UTRAN FDD – FDD Inter frequency handover: unknown target cell	870

A.5.1.5.1	Test Purpose and Environment	870

A.5.1.5.2	Test Requirements	872

A.5.1.6 	E-UTRAN TDD – TDD Inter frequency handover; unknown Target Cell	872

A.5.1.6.1	Test Purpose and Environment	872

A.5.1.6.2	Test Requirements	874

A.5.1.7	E-UTRAN FDD – TDD Inter frequency handover	874

A.5.1.7.1	Test Purpose and Environment	874

A.5.1.7.2	Test Requirements	877

A.5.1.8	E-UTRAN TDD – FDD Inter frequency handover	877

A.5.1.8.1	Test Purpose and Environment	877

A.5.1.8.2	Test Requirements	880

A.5.1.9	E-UTRAN FDD - FDD Intra frequency handover for 5MHz bandwidth	880

A.5.1.9.1	Test Purpose and Environment	880

A.5.1.9.2	Test Requirements	881

A.5.1.10	E-UTRAN FDD - FDD Intra frequency handover for UE category 0	881

A.5.1.10.1	Test Purpose and Environment	881

A.5.1.10.2	Test Requirements	883

A.5.1.11	E-UTRAN HD - FDD Intra frequency handover for UE category 0	883

A.5.1.11.1	Test Purpose and Environment	883

A.5.1.11.2	Test Requirements	885

A.5.1.12	E-UTRAN TDD - TDD Intra frequency handover for UE category 0	885

A.5.1.12.1	Test Purpose and Environment	885

A.5.1.12.2	Test Requirements	887

A.5.1.13	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	887

A.5.1.13.1	Test Purpose and Environment	887

A.5.1.13.2	Test Requirements	889

A.5.1.14	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	890

A.5.1.14.1	Test Purpose and Environment	890

A.5.1.14.2	Test Requirements	891

A.5.1.15	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA	892

A.5.1.15.1	Test Purpose and Environment	892

A.5.1.15.2	Test Requirements	893

A.5.1.16	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	894

A.5.1.16.1	Test Purpose and Environment	894

A.5.1.16.2	Test Requirements	895

A.5.1.17	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	896

A.5.1.17.1	Test Purpose and Environment	896

A.5.1.17.2	Test Requirements	897

A.5.1.18	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB	898

A.5.1.18.1	Test Purpose and Environment	898

A.5.1.18.2	Test Requirements	899

A.5.1.19	E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis	900

A.5.1.19.1	Test Purpose and Environment	900

A.5.1.19.2	Test Requirements	901

A.5.1.20	E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis	902

A.5.1.20.1	Test Purpose and Environment	902

A.5.1.20.2	Test Requirements	903

A.5.1.21	E-UTRAN FDD - FDD Intra frequency RACH-less handover	904

A.5.1.21.1	Test Purpose and Environment	904

A.5.1.21.2	Test Requirements	905

A.5.1.22	E-UTRAN TDD - TDD Intra frequency RACH-less handover	905

A.5.1.22.1	Test Purpose and Environment	905

A.5.1.22.2	Test Requirements	907

A.5.1.23	E-UTRAN FDD – FDD Inter frequency RACH-less handover	907

A.5.1.23.1	Test Purpose and Environment	907

A.5.1.23.2	Test Requirements	909

A.5.1.24	E-UTRAN TDD – TDD Inter frequency RACH-less handover	909

A.5.1.24.1	Test Purpose and Environment	909

A.5.1.24.2	Test Requirements	911

A.5.1.25	E-UTRAN FDD - FDD Intra frequency make-before-break handover	911

A.5.1.25.1	Test Purpose and Environment	911

A.5.1.25.2	Test Requirements	913

A.5.1.26	E-UTRAN TDD - TDD Intra frequency make-before-break handover	913

A.5.1.26.1	Test Purpose and Environment	913

A.5.1.26.2	Test Requirements	915

A.5.1.27	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeA	915

A.5.1.27.1	Test Purpose and Environment	915

A.5.1.27.2	Test Requirements	917

A.5.1.28	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeA	918

A.5.1.28.1	Test Purpose and Environment	918

A.5.1.28.2	Test Requirements	919

A.5.1.29	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeA	920

A.5.1.29.1	Test Purpose and Environment	920

A.5.1.29.2	Test Requirements	921

A.5.1.30	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeB	922

A.5.1.30.1	Test Purpose and Environment	922

A.5.1.30.2	Test Requirements	923

A.5.1.31	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeB	924

A.5.1.31.1	Test Purpose and Environment	924

A.5.1.31.2	Test Requirements	925

A.5.1.32	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeB	926

A.5.1.32.1	Test Purpose and Environment	926

A.5.1.32.2	Test Requirements	927

A.5.1.33	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	928

A.5.1.33.1	Test Purpose and Environment	928

A.5.1.13.2	Test Requirements	929

A.5.1.34	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	930

A.5.1.34.1	Test Purpose and Environment	930

A.5.1.34.2	Test Requirements	931

A.5.1.35	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	932

A.5.1.35.1	Test Purpose and Environment	932

A.5.1.35.2	Test Requirements	933

A.5.1.36	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	934

A.5.1.36.1	Test Purpose and Environment	934

A.5.1.36.2	Test Requirements	935

A.5.1.37	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	936

A.5.1.37.1	Test Purpose and Environment	936

A.5.1.37.2	Test Requirements	937

A.5.1.38	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	938

A.5.1.38.1	Test Purpose and Environment	938

A.5.1.38.2	Test Requirements	939

A.5.2	E-UTRAN Handover to other RATs	940

A.5.2.1	E-UTRAN FDD – UTRAN FDD Handover	940

A.5.2.1.1	Test Purpose and Environment	940

A.5.2.1.2	Test Requirements	941

A.5.2.2	E-UTRAN TDD - UTRAN FDD Handover	942

A.5.2.2.1	Test Purpose and Environment	942

A.5.2.2.2	Test Requirements	945

A.5.2.3 E-UTRAN FDD- GSM Handover	945

A.5.2.3.1	Test Purpose and Environment	945

A.5.2.3.2	Test Requirements	946

A.5.2.4	E-UTRAN TDD - UTRAN TDD Handover	947

A.5.2.4.1	Test Purpose and Environment	947

A.5.2.4.1.1	Void	947

A.5.2.4.1.2	1.28 Mcps TDD option	947

A.5.2.4.1.3	Void	949

A.5.2.4.2	Test Requirements	949

A.5.2.4.2.1	Void	949

A.5.2.4.2.2	1.28 Mcps TDD option	949

A.5.2.4.2.3	Void	949

A.5.2.5	E-UTRAN FDD – UTRAN TDD Handover	949

A.5.2.5.1	Test Purpose and Environment	949

A.5.2.5.1.3	Void	952

A.5.2.5.2	Test Requirements	952

A.5.2.5.2.1	Void	952

A.5.2.5.2.2	1.28 Mcps TDD option	952

A.5.2.5.2.3	Void	952

A.5.2.6	E-UTRAN TDD - GSM Handover	952

A.5.2.6.1	Test Purpose and Environment	952

A.5.2.6.2	Test Requirements	954

A.5.2.7	E-UTRAN FDD – UTRAN FDD Handover; Unknown Target Cell	955

A.5.2.7.1	Test Purpose and Environment	955

A.5.2.7.2	Test Requirements	957

A.5.2.8	E-UTRAN FDD - GSM Handover; Unknown Target Cell	957

A.5.2.8.1	Test Purpose and Environment	957

A.5.2.8.2	Test Requirements	958

A.5.2.9	E-UTRAN TDD - GSM Handover; Unknown Target Cell	959

A.5.2.9.1	Test Purpose and Environment	959

A.5.2.9.2	Test Requirements	960

A.5.2.10	E-UTRAN TDD to UTRAN TDD handover: unknown target cell	961

A.5.2.10.1	Test Purpose and Environment	961

A.5.2.10.2	Test Requirements	963

A.5.2.10A	E-UTRAN FDD – UTRAN FDD Multicarrier Handover with two target cells	963

A.5.2.10A.1	Test Purpose and Environment	963

A.5.2.10A.2	Test Requirements	966

A.5.2.10B	E-UTRAN TDD – UTRAN FDD Multicarrier Handover with two target cells	966

A.5.2.10B.1	Test Purpose and Environment	966

A.5.2.10B.2	Test Requirements	969

A.5.2.11	E-UTRAN FDD – UTRAN FDD Handover for 5MHz Bandwidth	969

A.5.2.11.1	Test Purpose and Environment	969

A.5.2.11.2	Test Requirements	969

A.5.3	E-UTRAN Handover to Non-3GPP RATs	970

A.5.3.1	E-UTRAN FDD – HRPD Handover	970

A.5.3.1.1	Test Purpose and Environment	970

A.5.3.1.2	Test Requirements	972

A.5.3.2	E-UTRAN FDD – cdma2000 1X Handover	972

A.5.3.2.1	Test Purpose and Environment	972

A.5.3.2.2	Test Requirements	975

A.5.3.3	E-UTRAN FDD – HRPD Handover; Unknown Target Cell	975

A.5.3.3.1	Test Purpose and Environment	975

A.5.3.3.2	Test Requirements	977

A.5.3.4	E-UTRAN FDD – cdma2000 1X Handover; Unknown Target cell	978

A.5.3.4.1	Test Purpose and Environment	978

A.5.3.4.2	Test Requirements	979

A.5.3.5	E-UTRAN TDD – HRPD Handover	980

A.5.3.5.1	Test Purpose and Environment	980

A.5.3.5.2	Test Requirements	982

A.5.3.6	E-UTRAN TDD – cdma2000 1X Handover	983

A.5.3.6.1	Test Purpose and Environment	983

A.5.3.6.2	Test Requirements	986

A.6	RRC Connection Control	986

A.6.1	RRC Re-establishment	986

A.6.1.1	E-UTRAN FDD Intra-frequency RRC Re-establishment	986

A.6.1.1.1	Test Purpose and Environment	986

A.6.1.1.2	Test Requirements	987

A.6.1.2	E-UTRAN FDD Inter-frequency RRC Re-establishment	988

A.6.1.2.1	Test Purpose and Environment	988

A.6.1.2.2	Test Requirements	989

A.6.1.3	E-UTRAN TDD Intra-frequency RRC Re-establishment	990

A.6.1.3.1	Test Purpose and Environment	990

A.6.1.3.2	Test Requirements	991

A.6.1.4	E-UTRAN TDD Inter-frequency RRC Re-establishment	992

A.6.1.4.1	Test Purpose and Environment	992

A.6.1.4.2	Test Requirements	993

A.6.1.5	E-UTRAN FDD Intra-frequency RRC Re-establishment for 5MHz bandwidth	994

A.6.1.5.1	Test Purpose and Environment	994

A.6.1.5.2	Test Requirements	994

A.6.1.6	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for UE category 0	994

A.6.1.6.1	Test Purpose and Environment	994

A.6.1.6.2	Test Requirements	996

A.6.1.7	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for UE category 0	997

A.6.1.7.1	Test Purpose and Environment	997

A.6.1.7.2	Test Requirements	998

A.6.1.8	E-UTRAN TDD Intra-frequency RRC Re-establishment for UE category 0	999

A.6.1.8.1	Test Purpose and Environment	999

A.6.1.8.2	Test Requirements	1000

A.6.1.9	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1001

A.6.1.9.1	Test Purpose and Environment	1001

A.6.1.9.2	Test Requirements	1002

A.6.1.10	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1003

A.6.1.10.1	Test Purpose and Environment	1003

A.6.1.10.2	Test Requirements	1004

A.6.1.11	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1005

A.6.1.11.1	Test Purpose and Environment	1005

A.6.1.11.2	Test Requirements	1006

A.6.1.12	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1007

A.6.1.12.1	Test Purpose and Environment	1007

A.6.1.12.2	Test Requirements	1008

A.6.1.13	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1009

A.6.1.13.1	Test Purpose and Environment	1009

A.6.1.13.2	Test Requirements	1010

A.6.1.14	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1011

A.6.1.14.1	Test Purpose and Environment	1011

A.6.1.14.2	Test Requirements	1012

A.6.1.15	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1013

A.6.1.15.1	Test Purpose and Environment	1013

A.6.1.15.2	Test Requirements	1015

A.6.1.16	HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1016

A.6.1.16.1	Test Purpose and Environment	1016

A.6.1.16.2	Test Requirements	1018

A.6.1.17	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1019

A.6.1.17.1	Test Purpose and Environment	1019

A.6.1.17.2	Test Requirements	1020

A.6.1.18	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1021

A.6.1.18.1	Test Purpose and Environment	1021

A.6.1.18.2	Test Requirements	1022

A.6.1.19	E-UTRAN TDD-TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1023

A.6.1.19.1	Test Purpose and Environment	1023

A.6.1.19.2	Test Requirements	1024

A.6.1.20	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1025

A.6.1.20.1	Test Purpose and Environment	1025

A.6.1.20.2	Test Requirements	1026

A.6.1.21	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1027

A.6.1.21.1	Test Purpose and Environment	1027

A.6.1.21.2	Test Requirements	1028

A.6.1.22	E-UTRAN TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1029

A.6.1.22.1	Test Purpose and Environment	1029

A.6.1.22.2	Test Requirements	1030

A.6.2	Random Access	1031

A.6.2.1	E-UTRAN FDD – Contention Based Random Access Test	1031

A.6.2.1.1	Test Purpose and Environment	1031

A.6.2.1.2	Test Requirements	1033

A.6.2.1.2.1	Random Access Response Reception	1033

A.6.2.1.2.2	No Random Access Response Reception	1033

A.6.2.1.2.3	Receiving a NACK on msg3	1033

A.6.2.1.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1033

A.6.2.1.2.5	Reception of a Correct Message over Temporary C-RNTI	1034

A.6.2.1.2.6	Contention Resolution Timer expiry	1034

A.6.2.2	E-UTRAN FDD – Non-Contention Based Random Access Test	1034

A.6.2.2.1	Test Purpose and Environment	1034

A.6.2.2.2	Test Requirements	1035

A.6.2.2.2.1	Random Access Response Reception	1036

A.6.2.2.2.2	No Random Access Response Reception	1036

A.6.2.3	E-UTRAN TDD – Contention Based Random Access Test	1036

A.6.2.3.1	Test Purpose and Environment	1036

A.6.2.3.2	Test Requirements	1038

A.6.2.3.2.1	Random Access Response Reception	1038

A.6.2.3.2.2	No Random Access Response reception	1038

A.6.2.3.2.3	Receiving a NACK on msg3	1038

A.6.2.3.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1039

A.6.2.3.2.5	Reception of a Correct Message over Temporary C-RNTI	1039

A.6.2.3.2.6	Contention Resolution Timer expiry	1039

A.6.2.4	E-UTRAN TDD – Non-Contention Based Random Access Test	1039

A.6.2.4.1	Test Purpose and Environment	1039

A.6.2.4.2	Test Requirements	1041

A.6.2.4.2.1	Random Access Response Reception	1041

A.6.2.4.2.2	No Random Access Response Reception	1041

A.6.2.5	E-UTRAN FDD – Contention Based Random Access Test for 5MHz bandwidth	1041

A.6.2.5.1	Test Purpose and Environment	1041

A.6.2.5.2	Test Requirements	1042

A.6.2.6	E-UTRAN FDD – Non-contention Based Random Access Test for 5MHz bandwidth	1042

A.6.2.6.1	Test Purpose and Environment	1042

A.6.2.6.2	Test Requirements	1042

A.6.2.7	E-UTRAN FDD – Non-Contention Based Random Access Test For SCell	1043

A.6.2.7.1	Test Purpose and Environment	1043

A.6.2.7.2	Test Requirements	1045

A.6.2.7.2.1	Random Access Response Reception	1045

A.6.2.7.2.2	No Random Access Response Reception	1045

A.6.2.7.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1045

A.6.2.8	E-UTRAN TDD – Non-Contention Based Random Access Test For SCell	1046

A.6.2.8.1	Test Purpose and Environment	1046

A.6.2.8.2	Test Requirements	1048

A.6.2.8.2.1	Random Access Response Reception	1048

A.6.2.8.2.2	No Random Access Response Reception	1048

A.6.2.8.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1048

A.6.2.9	3DL/3UL TDD CA Non-Contention Based Random Access Test for 2 SCells	1049

A.6.2.9.1	Test Purpose and Environment	1049

A.6.2.9.2	Test Requirements	1051

A.6.2.9.2.1	Random Access Response Reception	1051

A.6.2.9.2.2	No Random Access Response Reception	1052

A.6.2.9.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1052

A.6.2.10	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1053

A.6.2.10.1	Test Purpose and Environment	1053

A.6.2.10.2	Test Requirements	1055

A.6.2.10.2.1	Random Access Response Reception	1055

A.6.2.10.2.2	No Random Access Response Reception	1055

A.6.2.10.2.3	Receiving a NACK on msg3	1056

A.6.2.10.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1056

A.6.2.10.2.5	Reception of a Correct Message over Temporary C-RNTI	1056

A.6.2.10.2.6	Contention Resolution Timer expiry	1056

A.6.2.10.2.7	PRACH Resource Selection	1056

A.6.2.11	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1056

A.6.2.11.1	Test Purpose and Environment	1056

A.6.2.11.2	Test Requirements	1058

A.6.2.11.2.1	Random Access Response Reception	1058

A.6.2.11.2.2	No Random Access Response Reception	1059

A.6.2.11.2.3	Receiving a NACK on msg3	1059

A.6.2.11.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1059

A.6.2.11.2.5	Reception of a Correct Message over Temporary C-RNTI	1059

A.6.2.11.2.6	Contention Resolution Timer expiry	1059

A.6.2.11.2.7	PRACH Resource Selection	1060

A.6.2.12	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1060

A.6.2.12.1	Test Purpose and Environment	1060

A.6.2.12.2	Test Requirements	1062

A.6.2.12.2.1	Random Access Response Reception	1062

A.6.2.12.2.2	No Random Access Response Reception	1063

A.6.2.12.2.3	Receiving a NACK on msg3	1063

A.6.2.12.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1063

A.6.2.12.2.5	Reception of a Correct Message over Temporary C-RNTI	1063

A.6.2.12.2.6	Contention Resolution Timer expiry	1063

A.6.2.12.2.7	PRACH Resource Selection	1064

A.6.2.13	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1064

A.6.2.13.1	Test Purpose and Environment	1064

A.6.2.13.2	Test Requirements	1066

A.6.2.13.2.1	Random Access Response Reception	1066

A.6.2.13.2.2	No Random Access Response Reception	1066

A.6.2.13.2.3	Receiving a NACK on msg3	1067

A.6.2.13.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1067

A.6.2.13.2.5	Reception of a Correct Message over Temporary C-RNTI	1067

A.6.2.13.2.6	Contention Resolution Timer expiry	1067

A.6.2.13.2.7	PRACH Resource Selection	1067

A.6.2.14	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1067

A.6.2.14.1	Test Purpose and Environment	1067

A.6.2.14.2	Test Requirements	1069

A.6.2.14.2.1	Random Access Response Reception	1069

A.6.2.14.2.2	No Random Access Response Reception	1070

A.6.2.14.2.3	Receiving a NACK on msg3	1070

A.6.2.14.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1070

A.6.2.14.2.5	Reception of a Correct Message over Temporary C-RNTI	1070

A.6.2.14.2.6	Contention Resolution Timer expiry	1070

A.6.2.14.2.7	PRACH Resource Selection	1071

A.6.2.15	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1071

A.6.2.15.1	Test Purpose and Environment	1071

A.6.2.15.2	Test Requirements	1073

A.6.2.15.2.1	Random Access Response Reception	1073

A.6.2.15.2.2	No Random Access Response Reception	1074

A.6.2.15.2.3	Receiving a NACK on msg3	1074

A.6.2.15.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1074

A.6.2.15.2.5	Reception of a Correct Message over Temporary C-RNTI	1074

A.6.2.15.2.6	Contention Resolution Timer expiry	1074

A.6.2.15.2.7	PRACH Resource Selection	1075

A.6.2.16	Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1075

A.6.2.16.1	Test Purpose and Environment	1075

A.6.2.16.2	Test Requirements	1078

A.6.2.16.2.1	Random Access Response Reception	1078

A.6.2.16.2.2	No Random Access Response Reception	1079

A.6.2.16.2.3	Receiving a NACK on msg3	1079

A.6.2.16.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1079

A.6.2.16.2.5	Reception of a Correct Message over Temporary C-RNTI	1079

A.6.2.16.2.6	Contention Resolution Timer expiry	1079

A.6.2.16.2.7	NPRACH Resource Selection	1079

A.6.2.17	Contention Based Random Access Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1080

A.6.2.17.1	Test Purpose and Environment	1080

A.6.2.17.2	Test Requirements	1083

A.6.2.17.2.1	Random Access Response Reception	1083

A.6.2.17.2.2	No Random Access Response Reception	1084

A.6.2.17.2.3	Receiving a NACK on msg3	1084

A.6.2.17.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1084

A.6.2.17.2.5	Reception of a Correct Message over Temporary C-RNTI	1084

A.6.2.17.2.6	Contention Resolution Timer expiry	1084

A.6.2.17.2.7	NPRACH Resource Selection	1084

A.6.2.18	Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1085

A.6.2.18.1	Test Purpose and Environment	1085

A.6.2.18.2	Test Requirements	1088

A.6.2.18.2.1	Random Access Response Reception	1088

A.6.2.18.2.2	No Random Access Response Reception	1089

A.6.2.18.2.3	Receiving a NACK on msg3	1089

A.6.2.18.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1089

A.6.2.18.2.5	Reception of a Correct Message over Temporary C-RNTI	1089

A.6.2.18.2.6	Contention Resolution Timer expiry	1089

A.6.2.18.2.7	NPRACH Resource Selection	1089

A.6.3	RRC Connection Release with Redirection	1090

A.6.3.1	Redirection from E-UTRAN FDD to UTRAN FDD	1090

A.6.3.1.1	Test Purpose and Environment	1090

A.6.3.1.2	Test Requirements	1092

A.6.3.2	Redirection from E-UTRAN TDD to UTRAN FDD	1092

A.6.3.2.1	Test Purpose and Environment	1092

A.6.3.2.2	Test Requirements	1094

A.6.3.3	Redirection from E-UTRAN FDD to GERAN when System Information is provided	1094

A.6.3.3.1	Test Purpose and Environment	1094

A.6.3.3.2	Test Requirements	1095

A.6.3.4	Redirection from E-UTRAN TDD to GERAN when System Information is provided	1096

A.6.3.4.1	Test Purpose and Environment	1096

A.6.3.4.2	Test Requirements	1097

A.6.3.5	E-UTRA TDD RRC connection release redirection to UTRA TDD	1098

A.6.3.5.1	Test Purpose and Environment	1098

A.6.3.5.2	Test Requirements	1100

A.6.3.6	E-UTRA FDD RRC connection release redirection to UTRA TDD	1100

A.6.3.6.1	Test Purpose and Environment	1100

A. 6.3.6.2	Test Requirements	1102

A.6.3.7	E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided	1102

A.6.3.7.1	Test Purpose and Environment	1102

A.6.3.7.2	Test Requirements	1105

A.6.3.8	E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided	1105

A.6.3.8.1	Test Purpose and Environment	1105

A.6.3.8.2	Test Requirements	1108

A.6.3.9	Redirection from E-UTRAN FDD to UTRAN FDD without System Information	1108

A.6.3.9.1	Test Purpose and Environment	1108

A.6.3.9.2	Test Requirements	1110

A.6.3.10	Redirection from E-UTRAN FDD to GERAN when System Information is not provided	1110

A.6.3.10.1	Test Purpose and Environment	1110

A.6.3.10.2	Test Requirements	1112

A.6.3.11	Redirection from E-UTRAN TDD to GERAN when System Information is not provided	1112

A.6.3.11.1	Test Purpose and Environment	1112

A.6.3.11.2	Test Requirements	1114

A.6.3.12	E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided	1114

A.6.3.12.1	Test Purpose and Environment	1114

A.6.3.12.2	Test Requirements	1117

A.7	Timing and Signalling Characteristics	1118

A.7.1	UE Transmit Timing	1118

A.7.1.1	E-UTRAN FDD – UE Transmit Timing Accuracy Tests	1118

A.7.1.1.1	Test Purpose and Environment	1118

A.7.1.1.2	Test Requirements	1119

A.7.1.2	E-UTRAN TDD - UE Transmit Timing Accuracy Tests	1120

A.7.1.2.1	Test Purpose and Environment	1120

A.7.1.2.2	Test Requirements	1122

A.7.1.3	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell	1123

A.7.1.3.1	Test Purpose and Environment	1123

A.7.1.3.2	Test Requirements	1125

A.7.1.4	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell	1126

A.7.1.4.1	Test Purpose and Environment	1126

A.7.1.4.2	Test Requirements	1128

A.7.1.4A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell for 20 MHz + 10 MHz	1129

A.7.1.4A.1	Test Purpose and Environment	1129

A.7.1.4A.2	Test Requirements	1129

A.7.1.5	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for 5MHz Bandwidth	1129

A.7.1.5.1	Test Purpose and Environment	1129

A.7.1.5.2	Test Requirements	1130

A.7.1.6	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell in sTAG	1130

A.7.1.6.1	Test Purpose and Environment	1130

A.7.1.6.2	Test Requirements	1132

A.7.1.7	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG	1133

A.7.1.7.1	Test Purpose and Environment	1133

A.7.1.7.2	Test Requirements	1134

A.7.1.7A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +20MHz	1135

A.7.1.7A.1	Test Purpose and Environment	1135

A.7.1.7A.2	Test Requirements	1135

A.7.1.7B	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +10MHz	1135

A.7.1.7B.1	Test Purpose and Environment	1135

A.7.1.7B.2	Test Requirements	1135

A.7.1.8	Void	1136

A.7.1.8.1	Void	1136

A.7.1.8.2	Void	1136

A.7.1.9	Void	1136

A.7.1.9.1	Void	1136

A.7.1.9.2	Void	1136

A.7.1.10	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1136

A.7.1.10.1	Test Purpose and Environment	1136

A.7.1.10.2	Test Requirements	1138

A.7.1.11	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1138

A.7.1.11.1	Test Purpose and Environment	1138

A.7.1.11.2	Test Requirements	1140

A.7.1.12	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1140

A.7.1.12.1	Test Purpose and Environment	1140

A.7.1.12.2	Test Requirements	1142

A.7.1.13	3DL/3UL TDD CA UE Transmit Timing Accuracy Tests for 2 SCells	1142

A.7.1.13.1	Test Purpose and Environment	1142

A.7.1.13.2	Test Requirements	1144

A.7.1.14	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1145

A.7.1.14.1	Test Purpose and Environment	1145

A.7.1.14.2	Test Requirements	1146

A.7.1.15	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1146

A.7.1.15.1	Test Purpose and Environment	1146

A.7.1.15.2	Test Requirements	1147

A.7.1.16	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1148

A.7.1.16.1	Test Purpose and Environment	1148

A.7.1.16.2	Test Requirements	1149

A.7.1.17	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1150

A.7.1.17.1	Test Purpose and Environment	1150

A.7.1.17.2	Test Requirements	1152

A.7.1.18	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1153

A.7.1.18.1	Test Purpose and Environment	1153

A.7.1.18.2	Test Requirements	1155

A.7.1.19	E-UTRAN FDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1156

A.7.1.19.1	Test Purpose and Environment	1156

A.7.1.19.2	Test Requirements	1157

A.7.1.20	E-UTRAN TDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1157

A.7.1.20.1	Test Purpose and Environment	1157

A.7.1.20.2	Test Requirements	1159

A.7.1.21	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1159

A.7.1.21.1	Test Purpose and Environment	1159

A.7.1.21.2	Test Requirements	1161

A.7.1.22	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1161

A.7.1.22.1	Test Purpose and Environment	1161

A.7.1.22.2	Test Requirements	1163

A.7.1.23	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1163

A.7.1.23.1	Test Purpose and Environment	1163

A.7.1.23.2	Test Requirements	1165

A.7.1.24	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1165

A.7.1.24.1	Test Purpose and Environment	1165

A.7.1.24.2	Test Requirements	1166

A.7.1.25	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1167

A.7.1.25.1	Test Purpose and Environment	1167

A.7.1.25.2	Test Requirements	1168

A.7.1.26	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1169

A.7.1.26.1	Test Purpose and Environment	1169

A.7.1.26.2	Test Requirements	1170

A.7.2	UE Timing Advance	1171

A.7.2.1	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test	1171

A.7.2.1.1	Test Purpose and Environment	1171

A.7.2.1.2	Test Requirements	1173

A.7.2.2	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test	1173

A.7.2.2.1	Test Purpose and Environment	1173

A.7.2.2.2	Test Requirements	1175

A.7.2.3	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for 5MHz	1175

A.7.2.3.1	Test Purpose and Environment	1175

A.7.2.3.2	Test Requirements	1176

A.7.2.4	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for SCell in sTAG	1176

A.7.2.4.1	Test Purpose and Environment	1176

A.7.2.4.2	Test Requirements	1178

A.7.2.5	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG	1178

A.7.2.5.1	Test Purpose and Environment	1178

A.7.2.5.2	Test Requirements	1180

A.7.2.5A	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +20 MHz	1180

A.7.2.5A.1	Test Purpose and Environment	1180

A.7.2.5A.2	Test Requirements	1181

A.7.2.5B	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +10 MHz	1181

A.7.2.5B.1	Test Purpose and Environment	1181

A.7.2.5B.2	Test Requirements	1181

A.7.2.6	E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1181

A.7.2.6.1	Test Purpose and Environment	1181

A.7.2.6.2	Test Requirements	1184

A.7.2.7	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1184

A.7.2.7.1	Test Purpose and Environment	1184

A.7.2.7.2	Test Requirements	1186

A.7.2.8	E-UTRAN TDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1186

A.7.2.8.1	Test Purpose and Environment	1186

A.7.2.8.2	Test Requirements	1188

A.7.2.9.2	Test Requirements	1190

A.7.2.10	E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1190

A.7.2.10.1	Test Purpose and Environment	1190

A.7.2.10.2	Test Requirements	1191

A.7.2.11	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1191

A.7.2.11.1	Test Purpose and Environment	1191

A.7.2.11.2	Test Requirements	1193

A.7.2.12	E-UTRAN TDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1193

A.7.2.12.1	Test Purpose and Environment	1193

A.7.2.12.2	Test Requirements	1195

A.7.3	Radio Link Monitoring	1195

A.7.3.1	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync	1196

A.7.3.1.1	Test Purpose and Environment	1196

A.7.3.1.2	Test Requirements	1200

A.7.3.2	E-UTRAN FDD Radio Link Monitoring Test for In-sync	1200

A.7.3.2.1	Test Purpose and Environment	1200

A.7.3.2.2	Test Requirements	1204

A.7.3.3	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync	1204

A.7.3.3.1	Test Purpose and Environment	1204

A.7.3.3.2	Test Requirements	1208

A.7.3.4	E-UTRAN TDD Radio Link Monitoring Test for In-sync	1208

A.7.3.4.1	Test Purpose and Environment	1208

A.7.3.4.2	Test Requirements	1212

A.7.3.5	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX	1212

A.7.3.5.1	Test Purpose and Environment	1212

A.7.3.5.2	Test Requirements	1215

A.7.3.6	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX	1216

A.7.3.6.1	Test Purpose and Environment	1216

A.7.3.6.2	Test Requirements	1219

A.7.3.7	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX	1219

A.7.3.7.1	Test Purpose and Environment	1219

A.7.3.7.2	Test Requirements	1222

A.7.3.8	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX	1222

A.7.3.8.1	Test Purpose and Environment	1222

A.7.3.8.2	Test Requirements	1225

A.7.3.9 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction and Non-MBSFN ABS	1225

A.7.3.9.2	Test Requirements	1228

A.7.3.10	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1228

A.7.3.10.1	Test Purpose and Environment	1228

A.7.3.11	E-UTRAN FDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1232

A.7.3.11.1	Test Purpose and Environment	1232

A.7.3.11.2	Test Requirements	1237

A.7.3.12	E-UTRAN TDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1237

A.7.3.12.1	Test Purpose and Environment	1237

A.7.3.12.2	Test Requirements	1242

A.7.3.13 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1242

A.7.3.13.1	Test Purpose and Environment	1242

A.7.3.13.2	Test Requirements	1245

A.7.3.14 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1245

A.7.3.14.1	Test Purpose and Environment	1245

A.7.3.14.2	Test Requirements	1248

A.7.3.15	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1248

A.7.3.15.1	Test Purpose and Environment	1248

A.7.3.15.2	Test Requirements	1252

A.7.3.16	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1252

A.7.3.16.1	Test Purpose and Environment	1252

A.7.3.16.2	Test Requirements	1257

A.7.3.17	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1257

A.7.3.17.1	Test Purpose and Environment	1257

A.7.3.17.2	Test Requirements	1261

A.7.3.18	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1261

A.7.3.18.1	Test Purpose and Environment	1261

A.7.3.18.2	Test Requirements	1265

A.7.3.19	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1265

A.7.3.19.1	Test Purpose and Environment	1265

A.7.3.19.2	Test Requirements	1270

A.7.3.20	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1270

A.7.3.20.1	Test Purpose and Environment	1270

A.7.3.20.2	Test Requirements	1275

A.7.3.21	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1275

A.7.3.21.1	Test Purpose and Environment	1275

A.7.3.21.2	Test Requirements	1280

A.7.3.22	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1280

A.7.3.22.1	Test Purpose and Environment	1280

A.7.3.22.2	Test Requirements	1285

A.7.3.23	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth	1285

A.7.3.23.1	Test Purpose and Environment	1285

A.7.3.23.2	Test Requirements	1286

A.7.3.24	E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth	1286

A.7.3.24.1	Test Purpose and Environment	1286

A.7.3.24.2	Test Requirements	1287

A.7.3.25	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth	1287

A.7.3.25.1	Test Purpose and Environment	1287

A.7.3.25.2	Test Requirements	1288

A.7.3.26	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1288

A.7.3.26.1	Test Purpose and Environment	1288

A.7.3.26.2	Test Requirements	1291

A.7.3.27	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1291

A.7.3.27.1	Test Purpose and Environment	1291

A.7.3.27.2	Test Requirements	1294

A.7.3.28	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1294

A.7.3.28.1	Test Purpose and Environment	1294

A.7.3.28.2	Test Requirements	1297

A.7.3.29	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1297

A.7.3.29.1	Test Purpose and Environment	1297

A.7.3.29.2	Test Requirements	1300

A.7.3.30	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1300

A.7.3.30.1	Test Purpose and Environment	1300

A.7.3.30.2	Test Requirements	1303

A.7.3.31	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1303

A.7.3.31.1	Test Purpose and Environment	1303

A.7.3.31.2	Test Requirements	1306

A.7.3.32	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1306

A.7.3.32.1	Test Purpose and Environment	1306

A.7.3.32.2	Test Requirements	1309

A.7.3.33	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1309

A.7.3.33.1	Test Purpose and Environment	1309

A.7.3.33.2	Test Requirements	1312

A.7.3.34	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1312

A.7.3.34.1	Test Purpose and Environment	1312

A.7.3.34.2	Test Requirements	1315

A.7.3.35	E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE category 0	1315

A.7.3.35.1	Test Purpose and Environment	1315

A.7.3.35.2	Test Requirements	1318

A.7.3.36	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1318

A.7.3.36.1	Test Purpose and Environment	1318

A.7.3.36.2	Test Requirements	1321

A.7.3.37	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE category 0	1321

A.7.3.37.1	Test Purpose and Environment	1321

A.7.3.37.2	Test Requirements	1324

A.7.3.38	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1324

A.7.3.38.1	Test Purpose and Environment	1324

A.7.3.38.2	Test Requirements	1327

A.7.3.39	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC	1327

A.7.3.39.1	Test Purpose and Environment	1327

A.7.3.39.2	Test Requirements	1330

A.7.3.40	E-UTRAN TDD-TDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1331

A.7.3.40.1	Test Purpose and Environment	1331

A.7.3.40.2	Test Requirements	1333

A.7.3.41	E-UTRAN FDD-FDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1334

A.7.3.41.1	Test Purpose and Environment	1334

A.7.3.41.2	Test Requirements	1337

A.7.3.42	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for In-sync in DRX in asynchronous DC	1337

A.7.3.42.1	Test Purpose and Environment	1337

A.7.3.42.2	Test Requirements	1340

A.7.3.43	E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1340

A.7.3.43.1	Test Purpose and Environment	1340

A.7.3.43.2	Test Requirements	1343

A.7.3.44	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in FDD	1343

A.7.3.44.1	Test Purpose and Environment	1343

A.7.3.44.2	Test Requirements	1346

A.7.3.45	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in TDD	1347

A.7.3.45.1	Test Purpose and Environment	1347

A.7.3.45.2	Test Requirements	1349

A.7.3.46	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in FDD	1350

A.7.3.46.1	Test Purpose and Environment	1350

A.7.3.46.2	Test Requirements	1353

A.7.3.47	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in TDD	1353

A.7.3.47.1	Test Purpose and Environment	1353

A.7.3.47.2	Test Requirements	1356

A.7.3.48	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1356

A.7.3.48.1	Test Purpose and Environment	1356

A.7.3.48.2	Test Requirements	1359

A.7.3.49	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1359

A.7.3.49.1	Test Purpose and Environment	1359

A.7.3.49.2	Test Requirements	1362

A.7.3.50	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1362

A.7.3.50.1	Test Purpose and Environment	1362

A.7.3.50.2	Test Requirements	1365

A.7.3.51	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1365

A.7.3.51.1	Test Purpose and Environment	1365

A.7.3.51.2	Test Requirements	1368

A.7.3.52	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1368

A.7.3.52.1	Test Purpose and Environment	1368

A.7.3.52.2	Test Requirements	1371

A.7.3.53	E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1371

A.7.3.53.1	Test Purpose and Environment	1371

A.7.3.53.2	Test Requirements	1374

A.7.3.54	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1374

A.7.3.54.1	Test Purpose and Environment	1374

A.7.3.54.2	Test Requirements	1377

A.7.3.55	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1377

A.7.3.55.1	Test Purpose and Environment	1377

A.7.3.55.2	Test Requirements	1380

A.7.3.56	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1380

A.7.3.56.1	Test Purpose and Environment	1380

A.7.3.56.2	Test Requirements	1383

A.7.3.57	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1383

A.7.3.57.1	Test Purpose and Environment	1383

A.7.3.57.2	Test Requirements	1386

A.7.3.58	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1386

A.7.3.58.1	Test Purpose and Environment	1386

A.7.3.58.2	Test Requirements	1389

A.7.3.59	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1389

A.7.3.59.1	Test Purpose and Environment	1389

A.7.3.59.2	Test Requirements	1392

A.7.3.60	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1392

A.7.3.60.1	Test Purpose and Environment	1392

A.7.3.60.2	Test Requirements	1396

A.7.3.61	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1396

A.7.3.61.1	Test Purpose and Environment	1396

A.7.3.61.2	Test Requirements	1400

A.7.3.62	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1400

A.7.3.62.1	Test Purpose and Environment	1400

A.7.3.62.2	Test Requirements	1404

A.7.3.63	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1404

A.7.3.63.1	Test Purpose and Environment	1404

A.7.3.63.2	Test Requirements	1408

A.7.3.64	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1408

A.7.3.64.1	Test Purpose and Environment	1408

A.7.3.64.2	Test Requirements	1411

A.7.3.65	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1412

A.7.3.65.1	Test Purpose and Environment	1412

A.7.3.65.2	Test Requirements	1415

A.7.3.66	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1416

A.7.3.66.1	Test Purpose and Environment	1416

A.7.3.66.2	Test Requirements	1418

A.7.3.67	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1419

A.7.3.67.1	Test Purpose and Environment	1419

A.7.3.67.2	Test Requirements	1422

A.7.3.68	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1423

A.7.3.68.1	Test Purpose and Environment	1423

A.7.3.68.2	Test Requirements	1425

A.7.3.69	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1425

A.7.3.69.1	Test Purpose and Environment	1425

A.7.3.69.2	Test Requirements	1427

The rate of correct events observed during repeated tests shall be at least 90%.A.7.3.70	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1427

A.7.3.70.1	Test Purpose and Environment	1427

A.7.3.70.2	Test Requirements	1429

A.7.3.71	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1429

A.7.3.71.1	Test Purpose and Environment	1429

A.7.3.71.2	Test Requirements	1432

A.7.3.72	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1432

A.7.3.72.1	Test Purpose and Environment	1432

A.7.3.72.2	Test Requirements	1434

A.7.3.73	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1434

A.7.3.73.1	Test Purpose and Environment	1434

A.7.3.73.2	Test Requirements	1436

A.7.3.74	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1436

A.7.3.74.1	Test Purpose and Environment	1436

A.7.3.74.2	Test Requirements	1439

A.7.3.75	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1439

A.7.3.75.1	Test Purpose and Environment	1439

A.7.3.75.2	Test Requirements	1441

A.7.3.76	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1442

A.7.3.76.1	Test Purpose and Environment	1442

A.7.3.76.2	Test Requirements	1445

A.7.3.77	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1445

A.7.3.77.1	Test Purpose and Environment	1445

A.7.3.77.2	Test Requirements	1448

A.7.3.78	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1448

A.7.3.78.1	Test Purpose and Environment	1448

A.7.3.78.2	Test Requirements	1451

A.7.3.79	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1451

A.7.3.79.1	Test Purpose and Environment	1451

A.7.3.79.2	Test Requirements	1454

A.7.3.80	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1454

A.7.3.80.1	Test Purpose and Environment	1454

A.7.3.80.2	Test Requirements	1457

A.7.3.81	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1457

A.7.3.81.1	Test Purpose and Environment	1457

A.7.3.81.2	Test Requirements	1460

A.7.3.82	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1460

A.7.3.82.1	Test Purpose and Environment	1460

A.7.3.82.2	Test Requirements	1463

A.7.3.83	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1463

A.7.3.83.1	Test Purpose and Environment	1463

A.7.3.83.2	Test Requirements	1464

A.7.3.84	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1465

A.7.3.84.1	Test Purpose and Environment	1465

A.7.3.84.2	Test Requirements	1467

A.7.3.85	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1467

A.7.3.85.1	Test Purpose and Environment	1467

A.7.3.85.2	Test Requirements	1468

A.7.3.86	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1469

A.7.3.86.1	Test Purpose and Environment	1469

A.7.3.86.2	Test Requirements	1471

A.7.3.87	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1471

A.7.3.87.1	Test Purpose and Environment	1471

A.7.3.87.2	Test Requirements	1473

A.7.4	Interruption for Dual Connectivity	1474

A.7.4.1	E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1474

A.7.4.1.1	Test Purpose and Environment	1474

A.7.4.1.2	Test Requirements	1476

A.7.4.2	E-UTRAN TDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1476

A.7.4.2.1	Test Purpose and Environment	1476

A.7.4.2.2	Test Requirements	1478

A.7.4.3	E-UTRAN FDD-FDD Interruption at transitions between active and non-active during DRX in asynchronous dual connectivity	1478

A.7.4.3.1	Test Purpose and Environment	1478

A.7.4.3.2	Test Requirements	1480

A.7.4.4	E-UTRAN FDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1480

A.7.4.4.1	Test Purpose and Environment	1480

A.7.4.4.2	Test Requirements	1483

A.7.4.5	E-UTRAN TDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1483

A.7.4.5.1	Test Purpose and Environment	1483

A.7.4.5.2	Test Requirements	1485

A.7.4.6	E-UTRAN FDD-TDD DC interruption at SRS carrier based switching	1485

A.7.4.6.1	Test Purpose and Environment	1485

A.7.4.6.2	Test Requirements	1487

A.7.4.7	E-UTRAN TDD-TDD DC interruption at SRS carrier based switching	1487

A.7.4.7.1	Test Purpose and Environment	1487

A.7.4.7.2	Test Requirements	1491

A.7.5	Proximity-based Services	1491

A.7.5.1	E-UTRAN FDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1491

A.7.5.1.1	Test Purpose and Environment	1491

A.7.5.1.2	Test Requirements	1492

A.7.5.2	E-UTRAN TDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1493

A.7.5.2.1	Test Purpose and Environment	1493

A.7.5.1.2	Test Requirements	1493

A.7.5.3	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery	1494

A.7.5.3.1	Test Purpose and Environment	1494

A.7.5.3.2	Test Requirements	1495

A.7.5.4	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	1496

A.7.5.4.1	Test Purpose and Environment	1496

A.7.5.4.2	Test Requirements	1497

A.7.5.5	E-UTRAN FDD - Interruptions due to ProSe Direct Communication	1498

A.7.5.5.1	Test Purpose and Environment	1498

A.7.5.5.2	Test Requirements	1500

A.7.5.6	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery with discovery period less than 320ms	1501

A.7.5.6.1	Test Purpose and Environment	1501

A.7.5.6.2	Test Requirements	1502

A.7.5.7	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery	1503

A.7.5.7.1	Test Purpose and Environment	1503

A.7.5.7.2	Test Requirements	1505

A.7.5.8	E-UTRAN FDD-FDD - Cell reselection and timing accuracy for ProSe Direct Discovery transmission on non-serving frequency	1505

A.7.5.8.1	Test Purpose and Environment	1505

A.7.5.8.2	Test Requirements	1507

A.7.5.9	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery reception on non-serving frequency	1508

A.7.5.9.1	Test Purpose and Environment	1508

A.7.5.9.2	Test Requirements	1510

A.7.5.10	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery transmission on non-serving frequency	1511

A.7.5.10.1	Test Purpose and Environment	1511

A.7.5.10.2	Test Requirements	1513

A.7.5.11	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Communication on non-serving frequency	1514

A.7.5.11.1	Test Purpose and Environment	1514

A.7.5.11.2	Test Requirements	1516

A.7.5.12	E-UTRAN FDD - Selection / Reselection of ProSe relay UE	1516

A.7.5.12.1	Test Purpose and Environment	1516

A.7.5.12.2	Test Requirements	1520

A.7.6	Interruption for carrier aggregation	1521

A.7.6.1	E-UTRAN FDD-TDD CA interruption at SRS carrier based switching	1521

A.7.6.1.1	Test Purpose and Environment	1521

A.7.6.1.2	Test Requirements	1524

A.7.6.2	E-UTRAN TDD-TDD CA interruption at SRS carrier based switching	1524

A.7.6.2.1	Test Purpose and Environment	1524

A.7.6.2.2	Test Requirements	1527

A.8	UE Measurements Procedures	1527

A.8.1	E-UTRAN FDD Intra-frequency Measurements	1527

A.8.1.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1527

A.8.1.1.1	Test Purpose and Environment	1527

A.8.1.1.2	Test Requirements	1529

A.8.1.2	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1529

A.8.1.2.1	Test Purpose and Environment	1529

A.8.1.2.2	Test Requirements	1531

A.8.1.3	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1531

A.8.1.3.1	Test Purpose and Environment	1531

A.8.1.3.2	Test Requirements	1534

A.8.1.4	Void	1534

A.8.1.5	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1534

A.8.1.5.1	Test Purpose and Environment	1534

A.8.1.5.2	Test Requirements	1536

A.8.1.6	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1536

A.8.1.6.1	Test Purpose and Environment	1536

A.8.1.6.2	Test Requirements	1538

A.8.1.7 	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1538

A.8.1.7.1	Test Purpose and Environment	1538

A.8.1.7.2	Test Requirements	1540

A.8.1.8	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1541

A.8.1.8.1	Test Purpose and Environment	1541

A.8.1.8.2	Test Requirements	1543

A.8.1.9	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for 5MHz bandwidth	1544

A.8.1.9.1	Test Purpose and Environment	1544

A.8.1.9.2	Test Requirements	1544

A.8.1.10	E-UTRAN FDD-FDD Intra-Frequency Event Triggered Reporting under Fading Propagation Conditions in Synchronous Cells with DRX for 5 MHz Bandwidth	1544

A.8.1.10.1	Test Purpose and Environment	1544

A.8.1.10.2	Test Requirements	1545

A.8.1.11	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1545

A.8.1.11.1	Test Purpose and Environment	1545

A.8.1.11.2	Test Requirements	1547

A.8.1.12	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1548

A.8.1.12.1	Test Purpose and Environment	1548

A.8.1.12.2	Test Requirements	1549

A.8.1.13	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1550

A.8.1.13.1	Test Purpose and Environment	1550

A.8.1.13.2	Test Requirements	1552

A.8.1.14	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1552

A.8.1.14.1	Test Purpose and Environment	1552

A.8.1.14.2	Test Requirements	1554

A.8.1.15	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1555

A.8.1.15.1	Test Purpose and Environment	1555

A.8.1.15.2	Test Requirements	1556

A.8.1.16	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1557

A.8.1.16.1	Test Purpose and Environment	1557

A.8.1.16.2	Test Requirements	1559

A.8.1.17	Void	1559

A.8.1.18	Void	1559

A.8.1.19	E-UTRAN FDD-FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1559

A.8.1.19.1	Test Purpose and Environment	1559

A.8.1.19.2	Test Requirements	1561

A.8.1.20	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1562

A.8.1.20.1	Test Purpose and Environment	1562

A.8.1.20.2	Test Requirements	1564

A.8.1.21	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1564

A.8.1.21.1	Test Purpose and Environment	1564

A.8.1.21.2	Test Requirements	1566

A.8.1.22	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1567

A.8.1.22.1	Test Purpose and Environment	1567

A.8.1.22.2	Test Requirements	1569

A.8.1.23	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1569

A.8.1.23.1	Test Purpose and Environment	1569

A.8.1.23.2	Test Requirements	1570

A.8.1.24	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1571

A.8.1.24.1	Test Purpose and Environment	1571

A.8.1.24.2	Test Requirements	1573

A.8.1.25	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1573

A.8.1.25.1	Test Purpose and Environment	1573

A.8.1.25.2	Test Requirements	1575

A.8.1.26	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1575

A.8.1.26.1	Test Purpose and Environment	1575

A.8.1.27	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1578

A.8.1.27.1	Test Purpose and Environment	1578

A.8.1.27.2	Test Requirements	1580

A.8.1.28	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1580

A.8.1.28.1	Test Purpose and Environment	1580

A.8.1.28.2	Test Requirements	1582

A.8.1.29	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1582

A.8.1.29.1	Test Purpose and Environment	1582

A.8.1.29.2	Test Requirements	1584

A.8.1.30	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1585

A.8.1.30.1	Test Purpose and Environment	1585

A.8.1.30.2	Test Requirements	1587

A.8.1.31 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1587

A.8.1.31.1	Test Purpose and Environment	1587

A.8.1.31.2	Test Requirements	1589

A.8.1.32 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1590

A.8.1.32.1	Test Purpose and Environment	1590

A.8.1.32.2	Test Requirements	1591

A.8.1.33	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1592

A.8.1.33.1	Test Purpose and Environment	1592

A.8.1.33.2	Test Requirements	1593

A.8.1.34	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1594

A.8.1.34.1	Test Purpose and Environment	1594

A.8.1.34.2	Test Requirements	1595

A.8.1.35	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1596

A.8.1.35.1	Test Purpose and Environment	1596

A.8.1.35.2	Test Requirements	1597

A.8.1.36	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1598

A.8.1.36.1	Test Purpose and Environment	1598

A.8.1.36.2	Test Requirements	1599

A.8.1.37	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1600

A.8.1.37.1	Test Purpose and Environment	1600

A.8.1.37.2	Test Requirements	1602

A.8.1.38	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1602

A.8.1.38.1	Test Purpose and Environment	1602

A.8.1.38.2	Test Requirements	1603

A.8.1.39	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1604

A.8.1.39.1	Test Purpose and Environment	1604

A.8.1.39.2	Test Requirements	1606

A.8.1.40	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	1606

A.8.1.40.1	Test Purpose and Environment	1606

A.8.1.40.2	Test Requirements	1608

A.8.1.41	E-UTRAN FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1608

A.8.1.41.1	Test Purpose and Environment	1608

A.8.1.41.2	Test Requirement	1610

A.8.1.42	E-UTRAN HD-FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1610

A.8.1.42.1	Test Purpose and Environment	1610

A.8.1.42.2	Test Requirement	1611

A.8.2	E-UTRAN TDD Intra-frequency Measurements	1612

A.8.2.1	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1612

A.8.2.1.1	Test Purpose and Environment	1612

A.8.2.1.2	Test Requirements	1613

A.8.2.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1614

A.8.2.2.1	Test Purpose and Environment	1614

A.8.2.2.2	Test Requirements	1616

A.8.2.3	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1616

A.8.2.3.1	Test Purpose and Environment	1616

A.8.2.3.2	Test Requirements	1618

A.8.2.4	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1619

A.8.2.4.1	Test Purpose and Environment	1619

A.8.2.4.2	Test Requirements	1621

A.8.2.5	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1621

A.8.2.5.1	Test Purpose and Environment	1621

A.8.2.5.2	Test Requirements	1623

A.8.2.6	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1624

A.8.2.6.1	Test Purpose and Environment	1624

A.8.2.6.2	Test Requirements	1627

A.8.2.7	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1628

A.8.2.7.1	Test Purpose and Environment	1628

A.8.2.7.2	Test Requirements	1629

A.8.2.8	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1630

A.8.2.8.1	Test Purpose and Environment	1630

A.8.2.8.2	Test Requirements	1632

A.8.2.9	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1632

A.8.2.9.1	Test Purpose and Environment	1632

A.8.2.9.2	Test Requirements	1634

A.8.2.10	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1635

A.8.2.10.1	Test Purpose and Environment	1635

A.8.2.10.2	Test Requirements	1637

A.8.2.11	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	1637

A.8.2.11.1	Test Purpose and Environment	1637

A.8.2.11.2	Test Requirements	1639

A.8.2.12	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1639

A.8.2.12.1	Test Purpose and Environment	1639

A.8.2.12.2	Test Requirements	1641

A.8.2.13	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1642

A.8.2.13.1	Test Purpose and Environment	1642

A.8.2.13.2	Test Requirements	1644

A.8.2.14	E-UTRAN TDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1644

A.8.2.14.1	Test Purpose and Environment	1644

A.8.2.14.2	Test Requirement	1646

A.8.3	E-UTRAN FDD - FDD Inter-frequency Measurements	1647

A.8.3.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1647

A.8.3.1.1	Test Purpose and Environment	1647

A.8.3.1.2	Test Requirements	1648

A.8.3.2	E-UTRAN FDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	1649

A.8.3.2.1	Test Purpose and Environment	1649

A.8.3.2.2	Test Requirements	1651

A.8.3.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used	1651

A.8.3.3.1	Test Purpose and Environment	1651

A.8.3.3.2	Test Requirements	1654

A.8.3.4	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1654

A.8.3.4.1	Test Purpose and Environment	1654

A.8.3.4.2	Test Requirements	1656

A.8.3.5	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1656

A.8.3.5.2	Test Requirements	1658

A.8.3.6	E-UTRAN FDD-FDD Inter-frequency event triggered reporting without measurement gaps under AWGN propagation conditions in asynchronous cells	1658

A.8.3.6.1	Test Purpose and Environment	1658

A.8.3.6.2	Test Requirements	1660

A.8.3.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group	1661

A.8.3.7.1	Test Purpose and Environment	1661

A.8.3.7.2	Test Requirements	1663

A.8.3.8	FDD-FDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	1664

A.8.3.8.1	Test Purpose and Environment	1664

A.8.3.8.2	Test Requirements	1667

A.8.3.9	FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	1667

A.8.3.9.1	Test Purpose and Environment	1667

A.8.3.9.2	Test Requirements	1671

A.8.3.10	E-UTRAN FDD-FDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	1672

A.8.3.10.1	Test Purpose and Environment	1672

A.8.3.10.2	Test Requirements	1673

A.8.3.11	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells with burst gap	1674

A.8.3.11.1	Test Purpose and Environment	1674

A.8.3.11.2	Test Requirement	1675

A.8.3.12	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	1676

A.8.3.12.1	Test Purpose and Environment	1676

A.8.3.12.2	Test Requirement	1677

A.8.3.13	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	1678

A.8.3.13.1	Test Purpose and Environment	1678

A.8.3.13.2	Test Requirement	1679

A.8.3.14	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	1680

A.8.3.14.1	Test Purpose and Environment	1680

A.8.3.14.2	Test Requirement	1681

A.8.3.15	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	1682

A.8.3.15.1	Test Purpose and Environment	1682

A.8.3.15.2	Test Requirement	1683

A.8.3.16	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used	1684

A.8.3.16.1	Test Purpose and Environment	1684

A.8.3.16.2	Test Requirement	1686

A.8.3.17	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	1686

A.8.3.17.1	Test Purpose and Environment	1686

A.8.3.17.2	Test Requirement	1689

A.8.3.18	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	1689

A.8.3.18.1	Test Purpose and Environment	1689

A.8.3.18.2	Test Requirement	1692

A.8.3.19	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	1692

A.8.3.19.1	Test Purpose and Environment	1692

A.8.3.19.2	Test Requirement	1695

A.8.4	E-UTRAN TDD - TDD Inter-frequency Measurements	1695

A.8.4.1	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	1695

A.8.4.1.1	Test Purpose and Environment	1695

A.8.4.1.2	Test Requirements	1697

A.8.4.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells	1698

A.8.4.2.1	Test Purpose and Environment	1698

A.8.4.2.2	Test Requirements	1700

A.8.4.3	E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used	1700

A.8.4.3.1	Test Purpose and Environment	1700

A.8.4.3.2	Test Requirements	1702

A.8.4.4	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1703

A.8.4.4.1	Test Purpose and Environment	1703

A.8.4.4.2	Test Requirements	1705

A.8.4.5	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1706

A.8.4.5.1	Test Purpose and Environment	1706

A.8.4.5.2	Test Requirements	1708

A.8.4.6	E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0	1708

A.8.4.6.1	Test Purpose and Environment	1708

A.8.4.6.2	Test Requirements	1709

A.8.4.7	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for Increased Carrier Monitoring without Reduced Performance Group	1709

A.8.4.7.1	Test Purpose and Environment	1709

A.8.4.7.2	Test Requirements	1713

A.8.4.8	TDD-TDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	1713

A.8.4.8.1	Test Purpose and Environment	1713

A.8.4.8.2	Test Requirements	1716

A.8.4.9	TDD-TDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	1716

A.8.4.9.1	Test Purpose and Environment	1716

A.8.4.9.2	Test Requirements	1721

A.8.4.10	E-UTRAN TDD-TDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	1721

A.8.4.10.1	Test Purpose and Environment	1721

A.8.4.10.2	Test Requirements	1723

A.8.4.11	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells with burst gap	1723

A.8.4.11.1	Test Purpose and Environment	1723

A.8.4.11.2	Test Requirement	1725

A.8.4.12	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	1725

A.8.4.12.1	Test Purpose and Environment	1725

A.8.4.12.2	Test Requirement	1727

A.8.4.13	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	1728

A.8.4.13.1	Test Purpose and Environment	1728

A.8.4.13.2	Test Requirement	1729

A.8.4.14	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	1730

A.8.4.14.1	Test Purpose and Environment	1730

A.8.4.14.2	Test Requirement	1732

A.8.4.15	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	1732

A.8.4.15.1	Test Purpose and Environment	1732

A.8.4.15.2	Test Requirement	1735

A.8.5	E-UTRAN FDD - UTRAN FDD Measurements	1735

A.8.5.1	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	1735

A.8.5.1.1	Test Purpose and Environment	1735

A.8.5.1.2	Test Requirements	1737

A.8.5.2	E-UTRAN FDD - UTRAN FDD SON ANR cell search reporting under AWGN propagation conditions	1737

A.8.5.2.1	Test Purpose and Environment	1737

A.8.5.2.2	Test Requirements	1739

A.8.5.3	E-UTRAN FDD-UTRAN FDD event triggered reporting when DRX is used under fading propagation conditions	1739

A.8.5.3.1	Test Purpose and Environment	1739

A.8.5.3.2	Test Requirements	1742

A.8.5.4	E-UTRAN FDD - UTRAN FDD enhanced cell identification under AWGN propagation conditions	1742

A.8.5.4.1	Test Purpose and Environment	1742

A.8.5.4.2	Test Requirements	1744

A.8.5.5	E- UTRAN FDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	1744

A.8.5.5.1	Test Purpose and Environment	1744

A.8.5.5.2	Test Requirements	1747

A.8.5.6	E-UTRAN FDD - UTRAN FDD event triggered reporting without measurement gaps under AWGN propagation conditions	1747

A.8.5.6.1	Test Purpose and Environment	1747

A.8.5.6.2	Test Requirements	1748

A.8.5.7	E-UTRAN FDD - UTRAN FDD Event Triggered Reporting under Fading Propagation Conditions for 5 MHz Bandwidth	1749

A.8.5.7.1	Test Purpose and Environment	1749

A.8.5.7.2	Test Requirements	1749

A.8.5.8	E-UTRA FDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	1749

A.8.5.8.1	Test Purpose and Environment	1749

A.8.5.8.2	Test Requirements	1752

A.8.6	E-UTRAN TDD - UTRAN FDD Measurements	1752

A.8.6.1	E-UTRAN TDD - UTRAN FDD event triggered reporting under fading propagation conditions	1752

A.8.6.1.1	Test Purpose and Environment	1752

A.8.6.1.2	Test Requirements	1754

A.8.6.2	E- UTRAN TDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	1755

A.8.6.2.1	Test Purpose and Environment	1755

A.8.6.2.2	Test Requirements	1757

A.8.6.3	E-UTRA TDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	1757

A.8.6.3.1	Test Purpose and Environment	1757

A.8.6.3.2	Test Requirements	1760

A.8.7	E-UTRAN TDD – UTRAN TDD Measurements	1760

A.8.7.1	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	1760

A.8.7.1.1	Test Purpose and Environment	1760

A.8.7.1.1.1	Void	1760

A.8.7.1.1.2	1.28 Mcps TDD option	1760

A.8.7.1.1.3	Void	1762

A.8.7.1.2	Test Requirements	1762

A.8.7.1.2.1	Void	1762

A.8.7.1.2.2	1.28 Mcps TDD option	1762

A.8.7.1.2.3	Void	1762

A.8.7.2	E-UTRAN TDD-UTRAN TDD cell search when DRX is used under fading propagation conditions	1762

A.8.7.2.1	Test Purpose and Environment	1762

A.8.7.2.2	Test Requirements	1765

A.8.7.3	E-UTRAN TDD - UTRAN TDD SON ANR cell search reporting in AWGN propagation conditions	1766

A.8.7.3.1	Test Purpose and Environment	1766

A.8.7.3.2	Test Parameters	1766

A.8.7.3.3	Test Requirements	1767

A.8.7.4	E-UTRAN TDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	1768

A.8.7.4.1	Test Purpose and Environment	1768

A.8.7.4.2	Test Requirements	1770

A.8.7.5	E-UTRA TDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	1770

A.8.7.5.1	Test Purpose and Environment	1770

A.8.7.5.2	Test Requirements	1772

A.8.7A	TBD	1772

A.8.7A.1	E-UTRA FDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	1772

A.8.7A.1.1	Test Purpose and Environment	1772

A.8.7A.1.2	Test Requirements	1775

A.8.8	E-UTRAN FDD – GSM Measurements	1775

A.8.8.1	E-UTRAN FDD – GSM event triggered reporting in AWGN	1775

A.8.8.1.1	Test Purpose and Environment	1775

A.8.8.1.2	Test Requirements	1777

A.8.8.2	E-UTRAN FDD-GSM event triggered reporting when DRX is used in AWGN	1777

A.8.8.2.1	Test Purpose and Environment	1777

A.8.8.2.2	Test Requirements	1779

A.8.8.3	E-UTRAN FDD – GSM event triggered reporting in AWGN with enhanced BSIC identification	1780

A.8.8.3.1	Test Purpose and Environment	1780

A.8.8.3.2	Test Requirements	1781

A.8.9	E-UTRAN FDD - UTRAN TDD measurements	1782

A.8.9.1	E-UTRAN FDD - UTRAN TDD event triggered reporting in fading propagation conditions	1782

A.8.9.1.1	Test Purpose and Environment	1782

A.8.9.1.2	Test Requirements	1783

A.8.9.2	E-UTRAN FDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	1784

A.8.9.2.1	Test Purpose and Environment	1784

A.8.9.2.2	Test Requirements	1786

A.8.10	E-UTRAN TDD – GSM Measurements	1786

A.8.10.1	E-UTRAN TDD – GSM event triggered reporting in AWGN	1786

A.8.10.1.1	Test Purpose and Environment	1786

A.8.10.1.2	Test Requirements	1787

A.8.10.2	E-UTRAN TDD-GSM event triggered reporting when DRX is used in AWGN	1788

A.8.10.2.1	Test Purpose and Environment	1788

A.8.10.2.2	Test Requirements	1790

A.8.11	Monitoring of Multiple Layers	1790

A.8.11.1	Multiple E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions	1790

A.8.11.1.1	Test Purpose and Environment	1790

A.8.11.1.2	Test Requirements	1792

A.8.11.2	E-UTRAN TDD – E-UTRAN TDD and E-UTRAN TDD Inter-frequency event triggered reporting under fading propagation conditions	1793

A.8.11.2.1	Test Purpose and Environment	1793

A.8.11.2.2	Test Requirements	1794

A.8.11.3	E-UTRAN FDD-FDD Inter-frequency and UTRAN FDD event triggered reporting under fading propagation conditions	1794

A.8.11.3.1	Test Purpose and Environment	1794

A.8.11.3.2	Test Requirements	1797

A.8.11.4	InterRAT E-UTRA TDD to E-UTRA TDD and UTRA TDD cell search test case	1797

A.8.11.4.1	Test Purpose and Environment	1797

A.8.11.4.2	Test Requirements	1800

A.8.11.5	Combined E-UTRAN FDD – E-UTRA FDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	1800

A.8.11.5.1	Test Purpose and Environment	1800

A.8.11.5.2	Test Requirements	1802

A.8.11.6	Combined E-UTRAN TDD – E-UTRA TDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	1803

A.8.11.6.1	Test Purpose and Environment	1803

A.8.11.6.2	Test Requirements	1805

A.8.12	RSTD Intra-frequency Measurements	1806

A.8.12.1	E-UTRAN FDD intra-frequency RSTD measurement reporting delay test case	1806

A.8.12.1.1	Test Purpose and Environment	1806

A.8.12.1.2	Test Requirements	1810

A.8.12.1.2A	Test Requirements for UE Category 1bis	1810

A.8.12.2	E-UTRAN TDD intra-frequency RSTD measurement reporting delay test case	1810

A.8.12.2.1	Test Purpose and Environment	1810

A.8.12.2.2	Test Requirements	1815

A.8.12.2.2A	Test Requirements for UE Category 1bis	1815

A.8.12.3	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A	1815

A.8.12.3.1	Test Purpose and Environment	1815

A.8.12.3.2	Test Requirements	1821

A.8.12.4	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A	1821

A.8.12.4.1	Test Purpose and Environment	1821

A.8.12.4.2	Test Requirements	1826

A.8.12.5	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A	1826

A.8.12.5.1	Test Purpose and Environment	1826

A.8.12.5.2	Test Requirements	1831

A.8.12.6	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B	1831

A.8.12.6.1	Test Purpose and Environment	1831

A.8.12.6.2	Test Requirements	1836

A.8.12.7	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B	1836

A.8.12.7.1	Test Purpose and Environment	1836

A.8.12.7.2	Test Requirements	1841

A.8.12.8	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B	1841

A.8.12.8.1	Test Purpose and Environment	1841

A.8.12.8.2	Test Requirements	1846

A.8.13	RSTD Inter-frequency Measurements	1846

A.8.13.1	E-UTRAN FDD-FDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	1846

A.8.13.1.1	Test Purpose and Environment	1846

A.8.13.1.2	Test Requirements	1851

A.8.13.1.2A	Test Requirements for UE Category 1bis	1851

A.8.13.2	E-UTRAN TDD-TDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	1851

A.8.13.2.1	Test Purpose and Environment	1851

A.8.13.2.2	Test Requirements	1857

A.8.13.2.2A	Test Requirements for UE Category 1bis	1857

A.8.13.3	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A	1857

A.8.13.3.1	Test Purpose and Environment	1857

A.8.13.3.2	Test Requirements	1864

A.8.13.4	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A	1864

A.8.13.4.1	Test Purpose and Environment	1864

A.8.13.4.2	Test Requirements	1870

A.8.13.5	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A	1870

A.8.13.5.1	Test Purpose and Environment	1870

A.8.13.5.2	Test Requirements	1876

A.8.13.6	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B	1876

A.8.13.6.1	Test Purpose and Environment	1876

A.8.13.6.2	Test Requirements	1882

A.8.13.7	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B	1882

A.8.13.7.1	Test Purpose and Environment	1882

A.8.13.7.2	Test Requirements	1888

A.8.13.8	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B	1888

A.8.13.8.1	Test Purpose and Environment	1888

A.8.13.8.2	Test Requirements	1894

A.8.14	E-UTRAN TDD - FDD Inter-frequency Measurements	1894

A.8.14.1	E-UTRAN TDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1894

A.8.14.1.1	Test Purpose and Environment	1894

A.8.14.1.2	Test Requirements	1896

A.8.14.2	E-UTRAN TDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	1896

A.8.14.2.1	Test Purpose and Environment	1896

A.8.14.2.2	Test Requirements	1899

A.8.14.3	E-UTRAN TDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1899

A.8.14.3.1	Test Purpose and Environment	1899

A.8.14.3.2	Test Requirements	1901

A.8.15	E-UTRAN FDD - TDD Inter-frequency Measurements	1902

A.8.15.1	E-UTRAN FDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1902

A.8.15.1.1	Test Purpose and Environment	1902

A.8.15.1.2	Test Requirements	1903

A.8.15.2	E-UTRAN FDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	1903

A.8.15.2.1	Test Purpose and Environment	1903

A.8.15.2.2	Test Requirements	1906

A.8.15.3	E-UTRAN FDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1906

A.8.15.3.1	Test Purpose and Environment	1906

A.8.15.3.2	Test Requirements	1908

A.8.16	E-UTRAN Carrier Aggregation Measurements	1909

A.8.16.1	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX	1909

A.8.16.1.1	Test Purpose and Environment	1909

A.8.16.1.2	Test Requirements	1911

A.8.16.2	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX	1911

A.8.16.2.1	Test Purpose and Environment	1911

A.8.16.2.2	Test Requirements	1913

A.8.16.3	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	1913

A.8.16.3.1	Test Purpose and Environment	1913

A.8.16.3.2	Test Requirements	1915

A.8.16.3A	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with network controlled PCell interruption in non-DRX	1916

A.8.16.3A.1	Test Purpose and Environment	1916

A.8.16.3A.2	Test Requirements	1917

A.8.16.4	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell  with PCell interruption in non-DRX	1918

A.8.16.4.1	Test Purpose and Environment	1918

A.8.16.4.2	Test Requirements	1919

A.8.16.4A	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	1920

A.8.16.4A.1	Test Purpose and Environment	1920

A.8.16.4A.2	Test Requirements	1922

A.8.16.5	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	1923

A.8.16.5.1	Test Purpose and Environment	1923

A.8.16.5.2	Test Requirements	1923

A.8.16.6	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	1923

A.8.16.6.1	Test Purpose and Environment	1923

A.8.16.6.2	Test Requirements	1924

A.8.16.7	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	1924

A.8.16.7.1	 Test Purpose and Environment	1924

A.8.16.7.2	 Test Requirements	1925

A.8.16.8	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	1925

A.8.16.8.1	 Test Purpose and Environment	1925

A.8.16.8.2	 Test Requirements	1926

A.8.16.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	1926

A.8.16.9.1	Test Purpose and Environment	1926

A.8.16.9.2	Test Requirements	1927

A.8.16.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	1927

A.8.16.10.1	Test Purpose and Environment	1927

A.8.16.10.2	Test Requirements	1928

A.8.16.11	E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	1928

A.8.16.11.1	 Test Purpose and Environment	1928

A.8.16.11.2	 Test Requirements	1929

A.8.16.12	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	1929

A.8.16.12.1	 Test Purpose and Environment	1929

A.8.16.12.2	 Test Requirements	1929

A.8.16.13	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 5MHz +5 MHz bandwidth	1929

A.8.16.13.1	Test Purpose and Environment	1929

A.8.16.13.2	Test Requirements	1930

A.8.16.14	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 5 MHz +5 MHz bandwidth	1930

A.8.16.14.1	Test Purpose and Environment	1930

A.8.16.14.2	Test Requirements	1931

A.8.16.15	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5 +5 MHz bandwidth	1931

A.8.16.15.1	 Test Purpose and Environment	1931

A.8.16.7.2	 Test Requirements	1931

A.8.16.16	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5+5 MHz bandwidth	1931

A.8.16.16.1	 Test Purpose and Environment	1931

A.8.16.16.2	 Test Requirements	1932

A.8.16.17	E-UTRAN FDD activation and deactivation of known SCell in non-DRX	1932

A.8.16.17.1	Test Purpose and Environment	1932

A.8.16.17.2	Test Requirements	1934

A.8.16.17A	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 20MHz	1935

A.8.16.17A.1	Test Purpose and Environment	1935

A.8.16.17A.2	Test Requirements	1935

A.8.16.17B	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	1935

A.8.16.17B.1	Test Purpose and Environment	1935

A.8.16.17B.2	Test Requirements	1936

A.8.16.17C	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	1936

A.8.16.17C.1	Test Purpose and Environment	1936

A.8.16.17C.2	Test Requirements	1936

A.8.16.18	E-UTRAN TDD activation and deactivation of known SCell in non-DRX	1936

A.8.16.18.1	Test Purpose and Environment	1936

A.8.16.18.2	Test Requirements	1938

A.8.16.18A	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz	1939

A.8.16.18A.1	Test Purpose and Environment	1939

A.8.16.18A.2	Test Requirements	1939

A.8.16.18B	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	1939

A.8.16.18B.1	Test Purpose and Environment	1939

A.8.16.18B.2	Test Requirements	1940

A.8.16.18C	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	1940

A.8.16.18C.1	Test Purpose and Environment	1940

A.8.16.18C.2	Test Requirements	1940

A.8.16.18D	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz + 10MHz	1940

A.8.16.18D.1	Test Purpose and Environment	1940

A.8.16.18D.2	Test Requirements	1941

A.8.16.19	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX	1941

A.8.16.19.1	Test Purpose and Environment	1941

A.8.16.19.2	Test Requirements	1943

A.8.16.19A	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 20MHz	1943

A.8.16.19A.1	Test Purpose and Environment	1943

A.8.16.19A.2	Test Requirements	1944

A.8.16.19B 	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	1944

A.8.16.19B.1	Test Purpose and Environment	1944

A.8.16.19B.2	Test Requirements	1944

A.8.16.19C	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	1944

A.8.16.19C.1	Test Purpose and Environment	1944

A.8.16.19C.2	Test Requirements	1945

A.8.16.20	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX	1945

A.8.16.20.1	Test Purpose and Environment	1945

A.8.16.20.2	Test Requirements	1947

A.8.16.20A	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz	1948

A.8.16.20A.1	Test Purpose and Environment	1948

A.8.16.20A.2	Test Requirements	1948

A.8.16.20B	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	1948

A.8.16.20B.1	Test Purpose and Environment	1948

A.8.16.20B.2	Test Requirements	1949

A.8.16.20C	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	1949

A.8.16.20C.1	Test Purpose and Environment	1949

A.8.16.20C.2	Test Requirements	1949

A.8.16.20D	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz + 10MHz	1949

A.8.16.20D.1	Test Purpose and Environment	1949

A.8.16.20D.2	Test Requirements	1950

A.8.16.21	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20MHz+10MHz	1950

A.8.16.21.1	Test Purpose and Environment	1950

A.8.16.21.2	Test Requirements	1952

A.8.16.22	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 20MHz+10MHz	1952

A.8.16.22.1	Test Purpose and Environment	1952

A.8.16.22.2	Test Requirements	1954

A.8.16.23	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in FDD	1954

A.8.16.23.1	Test Purpose and Environment	1954

A.8.16.23.2	Test Requirements	1957

A.8.16.24	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in TDD	1957

A.8.16.24.1	Test Purpose and Environment	1957

A.8.16.24.2	Test Requirements	1960

A.8.16.25	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in FDD	1960

A.8.16.25.1	Test Purpose and Environment	1960

A.8.16.25.2	Test Requirements	1963

A.8.16.26	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in TDD	1963

A.8.16.26.1	Test Purpose and Environment	1963

A.8.16.26.2	Test Requirements	1966

A.8.16.27	3 DL PCell in FDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	1966

A.8.16.27.1	Test Purpose and Environment	1966

A.8.16.27.2	Test Requirements	1971

A.8.16.28	3 DL PCell in TDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	1971

A.8.16.28.1	Test Purpose and Environment	1971

A.8.16.28.2	Test Requirements	1976

A.8.16.29	3 DL FDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	1976

A.8.16.29.1	Test Purpose and Environment	1976

A.8.16.29.2	Test Requirements	1981

A.8.16.30	3 DL TDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	1981

A.8.16.30.1	Test Purpose and Environment	1981

A.8.16.30.2	Test Requirements	1986

A.8.16.31	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in FDD	1986

A.8.16.31.1	Test Purpose and Environment	1986

A.8.16.31.2	Test Requirements	1991

A.8.16.32	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in TDD	1991

A.8.16.32.1	Test Purpose and Environment	1991

A.8.16.32.2	Test Requirements	1996

A.8.16.33	E-UTRAN FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	1996

A.8.16.33.1	Test Purpose and Environment	1996

A.8.16.33.2	Test Requirements	2001

A.8.16.34	E-UTRAN TDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2001

A.8.16.34.1	Test Purpose and Environment	2001

A.8.16.34.2	Test Requirements	2006

A.8.16.35	3 DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX	2006

A.8.16.35.1	Test Purpose and Environment	2006

A.8.16.35.2	Test Requirements	2008

A.8.16.36	3 DL PCell in TDD CA Activation and Deactivation of Known SCell in Non-DRX	2009

A.8.16.36.1	Test Purpose and Environment	2009

A.8.16.36.2	Test Requirements	2011

A.8.16.37	3 DL FDD CA activation and deactivation of known SCell in non-DRX	2012

A.8.16.37.1	Test Purpose and Environment	2012

A.8.16.37.2	Test Requirements	2014

A.8.16.38	3 DL TDD CA activation and deactivation of known SCell in non-DRX	2015

A.8.16.38.1	Test Purpose and Environment	2015

A.8.16.38.2	Test Requirements	2017

A.8.16.39	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in FDD	2018

A.8.16.39.1	Test Purpose and Environment	2018

A.8.16.39.2	Test Requirements	2021

A.8.16.40	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in TDD	2021

A.8.16.40.1	Test Purpose and Environment	2021

A.8.16.40.2	Test Requirements	2024

A.8.16.41	3 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2024

A.8.16.41.1	Test Purpose and Environment	2024

A.8.16.41.2	Test Requirements	2026

A.8.16.42	3 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2027

A.8.16.42.1	Test Purpose and Environment	2027

A.8.16.42.2	Test Requirements	2029

A.8.16.43	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in FDD	2030

A.8.16.43.1	Test Purpose and Environment	2030

A.8.16.43.2	Test Requirements	2033

A.8.16.44	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in FDD	2033

A.8.16.44.1	Test Purpose and Environment	2033

A.8.16.44.2	Test Requirements	2036

A.8.16.45	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in TDD	2036

A.8.16.45.1	Test Purpose and Environment	2036

A.8.16.45.2	Test Requirements	2039

A.8.16.46	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in TDD	2039

A.8.16.46.1	Test Purpose and Environment	2039

A.8.16.46.2	Test Requirements	2041

A.8.16.47	2DL/2UL FDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2042

A.8.16.47.1	Test Purpose and Environment	2042

A.8.16.47.2	Test Requirements	2044

A.8.16.48	2DL/2UL TDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2045

A.8.16.48.1	Test Purpose and Environment	2045

A.8.16.48.2	Test Requirements	2047

A.8.16.49	2DL/2UL TDD-FDD CA (FDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2048

A.8.16.49.1	Test Purpose and Environment	2048

A.8.16.49.2	Test Requirements	2051

A.8.16.50	2DL/2UL TDD-FDD CA (TDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2051

A.8.16.50.1	Test Purpose and Environment	2051

A.8.16.50.2	Test Requirements	2053

A.8.16.51	E-UTRAN 4 DL FDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2054

A.8.16.51.1	Test Purpose and Environment	2054

A.8.16.51.2	Test Requirements	2059

A.8.16.52	E-UTRAN 4 DL TDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2059

A.8.16.52.1	Test Purpose and Environment	2059

A.8.16.52.2	Test Requirements	2064

A.8.16.53	4 DL PCell in FDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2064

A.8.16.53.1	Test Purpose and Environment	2064

A.8.16.53.2	Test Requirements	2068

A.8.16.54	4 DL PCell in TDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2068

A.8.16.54.1	Test Purpose and Environment	2068

A.8.16.54.2	Test Requirements	2072

A.8.16.55	E-UTRAN FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2072

A.8.16.55.1	Test Purpose and Environment	2072

A.8.16.55.2	Test Requirements	2078

A.8.16.56	E-UTRAN TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2078

A.8.16.56.1	Test Purpose and Environment	2078

A.8.16.56.2	Test Requirements	2084

A.8.16.57	E-UTRAN FDD 4DL CA activation and deactivation of know SCell in non-DRX	2084

A.8.16.57.1	Test Purpose and Environment	2084

A.8.16.57.2	Test Requirements	2086

A.8.16.58	E-UTRAN TDD 4DL CA activation and deactivation of know SCell in non-DRX	2087

A.8.16.58.1	Test Purpose and Environment	2087

A.8.16.58.2	Test Requirements	2089

A.8.16.59	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2090

A.8.16.59.1	Test Purpose and Environment	2090

A.8.16.59.2	Test Requirements	2094

A.8.16.60	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2095

A.8.16.60.1	Test Purpose and Environment	2095

A.8.16.60.2	Test Requirements	2098

A.8.16.61	E-UTRAN FDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2099

A.8.16.61.1	Test Purpose and Environment	2099

A.8.16.61.2	Test Requirements	2102

A.8.16.62	E-UTRAN TDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2102

A.8.16.62.1	Test Purpose and Environment	2102

A.8.16.62.2	Test Requirements	2104

A.8.16.63	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2105

A.8.16.63.1	Test Purpose and Environment	2105

A.8.16.63.2	Test Requirements	2108

A.8.16.64	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2109

A.8.16.64.1	Test Purpose and Environment	2109

A.8.16.64.2	Test Requirements	2113

A.8.16.65	5 DL FDD-TDD with PCell in FDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2113

A.8.16.65.1	Test Purpose and Environment	2113

A.8.16.65.2	Test Requirements	2118

A.8.16.66	5 DL FDD-TDD with PCell in TDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2118

A.8.16.66.1	Test Purpose and Environment	2118

A.8.16.66.2	Test Requirements	2123

A.8.16.67	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of Unknown SCell in non-DRX	2123

A.8.16.67.1	Test Purpose and Environment	2123

A.8.16.67.2	Test Requirements	2128

A.8.16.68	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of Unknown SCell in non-DRX	2128

A.8.16.68.1	Test Purpose and Environment	2128

A.8.16.68.2	Test Requirements	2133

A.8.16.69	5 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2133

A.8.16.69.1	Test Purpose and Environment	2133

A.8.16.69.2	Test Requirements	2137

A.8.16.70	5 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2137

A.8.16.70.1	Test Purpose and Environment	2137

A.8.16.70.2	Test Requirements	2141

A.8.16.71	5 DL FDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2141

A.8.16.71.1	Test Purpose and Environment	2141

A.8.16.71.2	Test Requirements	2146

A.8.16.72	5 DL TDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2146

A.8.16.72.1	Test Purpose and Environment	2146

A.8.16.72.2	Test Requirements	2151

A.8.16.73	5 DL FDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2151

A.8.16.73.1	Test Purpose and Environment	2151

A.8.16.73.2	Test Requirements	2157

A.8.16.74	5 DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2157

A.8.16.74.1	Test Purpose and Environment	2157

A.8.16.74.2	Test Requirements	2163

A.8.16.75	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of known SCell in non-DRX	2163

A.8.16.75.1	Test Purpose and Environment	2163

A.8.16.75.2	Test Requirements	2169

A.8.16.76	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of known SCell in non-DRX	2169

A.8.16.76.1	Test Purpose and Environment	2169

A.8.16.76.2	Test Requirements	2175

A.8.16.77	5 DL FDD CA activation and deactivation of know SCell in non-DRX	2175

A.8.16.77.1	Test Purpose and Environment	2175

A.8.16.77.2	Test Requirements	2179

A.8.16.78	5 DL TDD CA activation and deactivation of know SCell in non-DRX	2179

A.8.16.78.1	Test Purpose and Environment	2179

A.8.16.78.2	Test Requirements	2181

A.8.16.79	E-UTRAN PCell in FDD FDD-TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2182

A.8.16.79.1	Test Purpose and Environment	2182

A.8.16.79.2	Test Requirements	2188

A.8.16.80	E-UTRAN PCell in TDD TDD-FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2188

A.8.16.80.1	Test Purpose and Environment	2188

A.8.16.80.2	Test Requirements	2194

A.8.16.81	E-UTRAN PCell in FDD FDD-TDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2194

A.8.16.81.1	Test Purpose and Environment	2194

A.8.16.81.2	Test Requirements	2200

A.8.16.82	E-UTRAN PCell in TDD TDD-FDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2200

A.8.16.82.1	Test Purpose and Environment	2200

A.8.16.82.2	Test Requirements	2206

A.8.17	RSTD Measurements for E-UTRAN Carrier Aggregation	2206

A.8.17.1	E-UTRAN FDD RSTD measurement reporting delay test case	2206

A.8.17.1.1	Test Purpose and Environment	2206

A.8.17.1.2	Test Requirements	2212

A.8.17.2	E-UTRAN TDD RSTD measurement reporting delay test case	2212

A.8.17.2.1	Test Purpose and Environment	2212

A.8.17.2.2	Test Requirements	2219

A.8.17.3	E-UTRAN FDD RSTD Measurement Reporting Test Case for 20 MHz	2219

A.8.17.3.1	Test Purpose and Environment	2219

A.8.17.3.2	Test Requirements	2220

A.8.17.4	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20 MHz	2220

A.8.17.4.1	Test Purpose and Environment	2220

A.8.17.4.2	Test Requirements	2221

A.8.17.5	E-UTRAN FDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2221

A.8.17.5.1	Test Purpose and Environment	2221

A.8.17.5.2	Test Requirements	2222

A.8.17.6	E-UTRAN TDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2222

A.8.17.6.1	Test Purpose and Environment	2222

A.8.17.6.2	Test Requirements	2223

A.8.17.7	E-UTRAN FDD RSTD Measurement Reporting Test Case for 5 + 5 MHz Bandwidth	2224

A.8.17.7.1	Test Purpose and Environment	2224

A.8.17.7.2	Test Requirements	2224

A.8.17.8	E-UTRAN TDD RSTD Measurement Reporting Test Case for 5+5 MHz bandwidth	2225

A.8.17.8.1	Test Purpose and Environment	2225

A.8.17.8.2	Test Requirements	2225

A.8.17.9	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20MHz+10MHz	2226

A.8.17.9.1	Test Purpose and Environment	2226

A.8.17.9.2	Test Requirements	2227

A.8.17.10	E-UTRAN 3 DL FDD CA RSTD Measurement Reporting Delay Test Case	2227

A.8.17.10.1	Test Purpose and Environment	2227

A.8.17.10.2	Test Requirements	2234

A.8.17.11	E-UTRAN 3 DL TDD CA RSTD Measurement Reporting Delay Test Case	2235

A.8.17.11.1	Test Purpose and Environment	2235

A.8.17.11.2	Test Requirements	2241

A.8.18	E-UTRAN TDD – HRPD Measurements	2242

A.8.18.1	E-UTRAN TDD-HRPD event triggered reporting under fading propagation conditions	2242

A.8.18.1.1	Test Purpose and Environment	2242

A.8.18.1.2	Test Requirements	2244

A.8.19	E-UTRAN TDD – CDMA2000 1X Measurements	2244

A.8.19.1	E-UTRAN TDD – CDMA2000 1X event triggered reporting under fading propagation conditions	2244

A.8.19.1.1	Test Purpose and Environment	2244

A.8.19.1.2	Test Requirements	2245

A.8.20	Inter-frequency/RAT Measurements in CA mode	2246

A.8.20.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2246

A.8.20.1.1	Test Purpose and Environment	2246

A.8.20.1.2	Test Requirements	2247

A.8.20.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2248

A.8.20.2.1	Test Purpose and Environment	2248

A.8.20.2.2	Test Requirements	2249

A.8.20.2A	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth.	2250

A.8.20.2A.1	Test Purpose and Environment	2250

A.8.20.2A.2	Test Requirements	2250

A.8.20.2B	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +10 MHz bandwidth.	2250

A.8.20.2B.1	Test Purpose and Environment	2250

A.8.20.2B.2	Test Requirements	2253

A.8.20.3	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	2253

A.8.20.3.1	Test Purpose and Environment	2253

A.8.20.3.2	Test Requirements	2255

A.8.20.4	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	2255

A.8.20.4.1	Test Purpose and Environment	2255

A.8.20.4.1.1	1.28 Mcps TDD option	2255

A.8.20.4.2	Test Requirements	2257

A.8.20.4.2.1	1.28 Mcps TDD option	2257

A.8.20.4A	E-UTRAN TDD with 20 MHz +20 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2257

A.8.20.4A.1	Test Purpose and Environment	2257

A.8.20.4A.1.1	1.28 Mcps TDD option	2257

A.8.20.4A.2	Test Requirements	2257

A.8.20.4A.2.1	1.28 Mcps TDD option	2257

A.8.20.4B	E-UTRAN TDD with 20 MHz +10 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2258

A.8.20.4B.1	Test Purpose and Environment	2258

A.8.20.4B.1.1	1.28 Mcps TDD option	2258

A.8.20.4B.2	Test Requirements	2260

A.8.20.4B.2.1	1.28 Mcps TDD option	2260

A.8.21	CSG Proximity Indication Testing Case for E-UTRAN FDD – FDD Inter frequency	2260

A.8.21.1	Test Purpose and Environment	2260

A.8.21.2 Test Requirements	2264

A.8.22	E-UTRAN Discovery Signal Measurements	2264

A.8.22.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2264

A.8.22.1.1	Test Purpose and Environment	2264

A.8.22.1.2	Test Requirements	2267

A.8.22.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2267

A.8.22.2.1	Test Purpose and Environment	2267

A.8.22.2.2	Test Requirements	2270

A.8.22.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2270

A.8.22.3.1	Test Purpose and Environment	2270

A.8.22.3.2	Test Requirements	2273

A.8.22.4	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2273

A.8.22.4.1	Test Purpose and Environment	2273

A.8.22.4.2	Test Requirements	2276

A.8.22.5	E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2276

A.8.22.5.1	Test Purpose and Environment	2276

A.8.22.5.2	Test Requirements	2280

A.8.22.6	E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2280

A.8.22.6.1	Test Purpose and Environment	2280

A.8.22.6.2	Test Requirements	2284

A.8.22.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2284

A.8.22.7.1	Test Purpose and Environment	2284

A.8.22.7.2	Test Requirements	2288

A.8.22.8	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation condition in DRX based on CSI-RS based discovery signal	2288

A.8.22.8.1	Test Purpose and Environment	2288

A.8.22.8.2	Test Requirements	2292

A.8.22.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2292

A.8.22.9.1	Test Purpose and Environment	2292

A.8.22.9.2	Test Requirements	2294

A.8.22.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2295

A.8.22.10.1	Test Purpose and Environment	2295

A.8.22.10.2	Test Requirements	2297

A.8.22.11	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2298

A.8.22.11.1	Test Purpose and Environment	2298

A.8.22.11.2	Test Requirements	2301

A.8.22.12	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2301

A.8.22.12.1	Test Purpose and Environment	2301

A.8.22.12.2	Test Requirements	2305

A.8.23	E-UTRAN Dual Connectivity Measurements	2305

A.8.23.1	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2305

A.8.23.1.1	Test Purpose and Environment	2305

A.8.23.1.2	Test Requirements	2308

A.8.23.2	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in asynchronous DC	2308

A.8.23.2.1	Test Purpose and Environment	2308

A.8.23.2.2	Test Requirements	2311

A.8.23.3	E-UTRAN TDD-TDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2311

A.8.23.3.1	Test Purpose and Environment	2311

A.8.23.3.2	Test Requirements	2314

A.8.23.4	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2314

A.8.23.4.1	Test Purpose and Environment	2314

A.8.23.4.2	Test Requirements	2317

A.8.23.5	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in asynchronous DC	2317

A.8.23.5.1	Test Purpose and Environment	2317

A.8.23.5.2	Test Requirements	2320

A.8.23.6	E-UTRAN TDD-TDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2320

A.8.23.6.1	Test Purpose and Environment	2320

A.8.23.6.2	Test Requirements	2323

A.8.23.7	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Synchronous DC	2323

A.8.23.7.1	Test Purpose and Environment	2323

A.8.23.7.2	Test Requirements	2325

A.8.23.8	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Asynchronous DC	2326

A.8.23.8.1	Test Purpose and Environment	2326

A.8.23.8.2	Test Requirements	2328

A.8.23.9	E-UTRAN TDD Addition and Release Delay of known PSCell in Synchronous DC	2329

A.8.23.9.1	Test Purpose and Environment	2329

A.8.23.9.2	Test Requirements	2332

A.8.23.10	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2332

A.8.23.10.1	Test Purpose and Environment	2332

A.8.23.10.2	Test Requirements	2335

A.8.23.11	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2335

A.8.23.11.1	Test Purpose and Environment	2335

A.8.23.11.2	Test Requirements	2338

A.8.23.12	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2338

A.8.23.12.1	Test Purpose and Environment	2338

A.8.23.12.2	Test Requirements	2341

A.8.23.13	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2341

A.8.23.13.1	Test Purpose and Environment	2341

A.8.23.13.2	Test Requirements	2344

A.8.23.14	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in FDD	2344

A.8.23.14.1	Test Purpose and Environment	2344

A.8.23.14.2	Test Requirements	2347

A.8.23.15	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in TDD	2347

A.8.23.15.1	Test Purpose and Environment	2347

A.8.23.15.2	Test Requirements	2350

A.8.23.16	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with no DRX in asynchronous DC	2350

A.8.23.16.1	Test Purpose and Environment	2350

A.8.23.16.2	Test Requirements	2351

A.8.23.17	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with DRX in asynchronous DC	2352

A.8.23.17.1	Test Purpose and Environment	2352

A.8.23.17.2	Test Requirements	2354

A.8.23.18	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2354

A.8.23.18.1	Test Purpose and Environment	2354

A.8.23.18.2	Test Requirements	2355

A.8.23.19	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	2356

A.8.23.19.1	Test Purpose and Environment	2356

A.8.23.19.2	Test Requirements	2357

A.8.23.20	E-UTRAN TDD - TDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2358

A.8.23.20.1	Test Purpose and Environment	2358

A.8.23.20.2	Test Requirements	2359

A.8.23.21	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2360

A.8.23.21.1	Test Purpose and Environment	2360

A.8.23.21.2	Test Requirements	2361

A.8.23.22	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	2362

A.8.23.22.1	Test Purpose and Environment	2362

A.8.23.22.2	Test Requirements	2364

A.8.23.23	E-UTRAN TDD - TDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2364

A.8.23.23.1	Test Purpose and Environment	2364

A.8.23.23.2	Test Requirements	2366

A.8.23.24	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	2367

A.8.23.24.1	Test Purpose and Environment	2367

A.8.23.24.2	Test Requirements	2369

A.8.23.25	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in asynchronous DC	2370

A.8.23.25.1	Test Purpose and Environment	2370

A.8.23.25.2	Test Requirements	2372

A.8.23.26	E-UTRAN TDD-TDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	2373

A.8.23.26.1	Test Purpose and Environment	2373

A.8.23.26.2	Test Requirements	2375

A.8.23.27	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	2376

A.8.23.27.1	Test Purpose and Environment	2376

A.8.23.27.2	Test Requirements	2379

A.8.23.28	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in asynchronous DC	2379

A.8.23.28.1	Test Purpose and Environment	2379

A.8.23.28.2	Test Requirements	2383

A.8.23.29	E-UTRAN TDD-TDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	2383

A.8.23.29.1	Test Purpose and Environment	2383

A.8.23.29.2	Test Requirements	2387

A.8.24	Proximity-based Services	2387

A.8.24.1	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	2387

A.8.24.1.1	Test Purpose and Environment	2387

A.8.24.1.2	Test Requirements	2388

A.8.24.2	E-UTRAN TDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	2389

A.8.24.2.1	Test Purpose and Environment	2389

A.8.24.2.2	Test Requirements	2390

A.8.24.3	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Communication	2390

A.8.24.3.1	Test Purpose and Environment	2391

A.8.24.3.2	Test Requirements	2392

A.8.25	E-UTRAN-WLAN Measurements	2393

A.8.25.1	E-UTRAN FDD-WLAN Event Triggered Reporting in non-DRX under AWGN	2393

A.8.25.1.1	Test Purpose and Environment	2393

A.8.25.1.2	Test Requirements	2395

A.8.25.2	E-UTRAN TDD-WLAN Event Triggered Reporting in non-DRX under AWGN	2395

A.8.25.2.1	Test Purpose and Environment	2395

A.8.25.2.2	Test Requirements	2397

A.8.26 Frame Structure 3 (FS3)	2397

A.8.26.1	E-UTRAN FDD-FS3 Activation and deactivation of known FS3 SCell with FDD PCell in non-DRX	2397

A.8.26.1.1	Test Purpose and Environment	2397

A.8.26.1.2	Test Requirements	2399

A.8.26.2	E-UTRAN TDD-FS3 Activation and deactivation of known FS3 SCell with TDD PCell in non-DRX	2400

A.8.26.2.1	Test Purpose and Environment	2400

A.8.26.2.2	Test Requirements	2402

A.8.26.3	E-UTRAN FDD-FS3 Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2403

A.8.26.3.1	Test Purpose and Environment	2403

A.8.26.3.2	Test Requirements	2406

A.8.26.3A	E-UTRAN FDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2406

A.8.26.3A.1	Test Purpose and Environment	2406

A.8.26.3A.2	Test Requirements	2409

A.8.26.4	E-UTRAN TDD-FS3 Event triggered reporting on deactivated FS3 SCell and TDD PCell interruption in non-DRX	2409

A.8.26.4.1	Test Purpose and Environment	2409

A.8.26.4.2	Test Requirements	2413

A.8.26.4A	E-UTRAN TDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2413

A.8.26.4A.1	Test Purpose and Environment	2413

A.8.26.4A.2	Test Requirements	2417

A.8.26.5	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	2417

A.8.26.5.1	Test Purpose and Environment	2417

A.8.26.5.2	Test Requirements	2421

A.8.26.5A	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	2421

A.8.26.5A.1	Test Purpose and Environment	2421

A.8.26.5A.2	Test Requirements	2424

A.8.26.6	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	2424

A.8.26.6.1	Test Purpose and Environment	2424

A.8.26.6.2	Test Requirements	2428

A.8.26.6A	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	2428

A.8.26.6A.1	Test Purpose and Environment	2428

A.8.26.6A.2	Test Requirements	2432

A.8.26.7	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	2432

A.8.26.7.1	Test Purpose and Environment	2432

A.8.26.7.2	Test Requirements	2435

A.8.26.8	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	2435

A.8.26.8.1	Test Purpose and Environment	2435

A.8.26.8.2	Test Requirements	2439

A.8.26.9	E-UTRAN FDD-FS3 Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2439

A.8.26.9.1	Test Purpose and Environment	2439

A.8.26.9.2	Test Requirements	2442

A.8.26.10	E-UTRAN TDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2442

A.8.26.10.1	Test Purpose and Environment	2442

A.8.26.10.2	Test Requirements	2445

A.9	Measurement Performance Requirements	2446

A.9.1	RSRP	2446

A.9.1.1	FDD Intra frequency case	2446

A.9.1.1.1	Test Purpose and Environment	2446

A.9.1.1.2	Test parameters	2446

A.9.1.1.3	Test Requirements	2448

A.9.1.2	TDD Intra frequency case	2448

A.9.1.2.1	Test Purpose and Environment	2448

A.9.1.2.2	Test parameters	2448

A.9.1.2.3	Test Requirements	2449

A.9.1.3	FDD—FDD Inter frequency case	2450

A.9.1.3.1	Test Purpose and Environment	2450

A.9.1.3.2	Test parameters	2450

A.9.1.3.3	Test Requirements	2452

A.9.1.4	TDD—TDD Inter frequency case	2452

A.9.1.4.1	Test Purpose and Environment	2452

A.9.1.4.2	Test parameters	2452

A.9.1.4.3	Test Requirements	2454

A.9.1.5	FDD—TDD Inter frequency case	2455

A.9.1.5.1	Test Purpose and Environment	2455

A.9.1.5.2	Test parameters	2455

A.9.1.5.3	Test Requirements	2456

A.9.1.6	FDD RSRP for E-UTRAN Carrier Aggregation	2457

A.9.1.6.1	Test Purpose and Environment	2457

A.9.1.6.2	Test parameters	2457

A.9.1.6.3	Test Requirements	2459

A.9.1.7	TDD RSRP for E-UTRAN Carrier Aggregation	2459

A.9.1.7.1	Test Purpose and Environment	2459

A.9.1.7.2	Test parameters	2459

A.9.1.7.3	Test Requirements	2461

A.9.1.8	FDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2461

A.9.1.8.1	Test Purpose and Environment	2461

A.9.1.8.2	Test parameters	2461

A.9.1.8.3	Test Requirements	2464

A.9.1.9	TDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2464

A.9.1.9.1	Test Purpose and Environment	2464

A.9.1.9.2	Test parameters	2464

A.9.1.9.3	Test Requirements	2467

A.9.1.10	FDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	2467

A.9.1.10.1	Test Purpose and Environment	2467

A.9.1.10.2	Test parameters	2467

A.9.1.10.3	Test Requirements	2469

A.9.1.11	TDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	2469

A.9.1.11.1	Test Purpose and Environment	2469

A.9.1.11.2	Test parameters	2469

A.9.1.11.3	Test Requirements	2472

A.9.1.12	FDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	2472

A.9.1.12.1	Test Purpose and Environment	2472

A.9.1.12.2	Test parameters	2472

A.9.1.12.3	Test Requirements	2473

A.9.1.13	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	2473

A.9.1.13.1	Test Purpose and Environment	2473

A.9.1.13.2	Test parameters	2473

A.9.1.13.3	Test Requirements	2474

A.9.1.14	FDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2474

A.9.1.14.1	Test Purpose and Environment	2474

A.9.1.14.2	Test parameters	2474

A.9.1.14.3	Test Requirements	2477

A.9.1.15	TDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2477

A.9.1.15.1	Test Purpose and Environment	2477

A.9.1.15.2	Test parameters	2477

A.9.1.15.3	Test Requirements	2479

A.9.1.16	FDD Intra frequency case for 5MHz Bandwidth	2480

A.9.1.16.1	Test Purpose and Environment	2480

A.9.1.16.2	Test parameters	2480

A.9.1.16.3	Test Requirements	2481

A.9.1.17	FDD—FDD Inter frequency case for 5MHz Bandwidth	2481

A.9.1.17.1	Test Purpose and Environment	2481

A.9.1.17.2	Test parameters	2482

A.9.1.17.3	Test Requirements	2483

A.9.1.18	FDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	2483

A.9.1.18.1	Test Purpose and Environment	2483

A.9.1.18.2	Test parameters	2483

A.9.1.18.3	Test Requirements	2484

A.9.1.19	TDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	2484

A.9.1.19.1	Test Purpose and Environment	2485

A.9.1.19.2	Test parameters	2485

A.9.1.19.3	Test Requirements	2485

A.9.1.20	FDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	2485

A.9.1.20.1	Test Purpose and Environment	2485

A.9.1.20.2	Test parameters	2485

A.9.1.20.3	Test Requirements	2486

A.9.1.21	TDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	2487

A.9.1.21.1	Test Purpose and Environment	2487

A.9.1.21.2	Test parameters	2487

A.9.1.21.3	Test Requirements	2487

A.9.1.22	RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	2487

A.9.1.22.1	Test Purpose and Environment	2487

A.9.1.22.2	Test parameters	2488

A.9.1.22.3	Test Requirements	2491

A.9.1.23 RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	2491

A.9.1.23.1	Test Purpose and Environment	2491

A.9.1.23.2	Test parameters	2491

A.9.1.23.3	Test Requirements	2493

A.9.1.24	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz + 10MHz	2494

A.9.1.24.1	Test Purpose and Environment	2494

A.9.1.24.2	Test parameters	2494

A.9.1.24.3	Test Requirements	2494

A.9.1.25	FDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2495

A.9.1.25.1	Test Purpose and Environment	2495

A.9.1.25.2	Test parameters	2495

A.9.1.25.3	Test Requirements	2497

A.9.1.26	TDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2497

A.9.1.26.1	Test Purpose and Environment	2497

A.9.1.26.2	Test parameters	2497

A.9.1.26.3	Test Requirements	2499

A.9.1.27	FDD—FDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2499

A.9.1.27.1	Test Purpose and Environment	2499

A.9.1.27.2	Test parameters	2499

A.9.1.27.3	Test Requirements	2501

A.9.1.28	TDD—TDD inter-frequency absolute and relative  RSRP accuracies in CRS based discovery signal	2501

A.9.1.28.1	Test Purpose and Environment	2501

A.9.1.28.2	Test parameters	2501

A.9.1.28.3	Test Requirements	2503

A.9.1.29	FDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2503

A.9.1.29.1	Test Purpose and Environment	2503

A.9.1.29.2	Test parameters	2503

A.9.1.29.3	Test Requirements	2505

A.9.1.30	TDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2505

A.9.1.30.1	Test Purpose and Environment	2505

A.9.1.30.2	Test parameters	2505

A.9.1.30.3	Test Requirements	2507

A.9.1.31	FDD—FDD inter-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2507

A.9.1.31.1	Test Purpose and Environment	2507

A.9.1.31.2	Test parameters	2507

A.9.1.31.3	Test Requirements	2509

A.9.1.32	TDD—TDD inter-frequency absolute and relative  CSI-RSRP accuracies in CSI-RS based discovery signal	2509

A.9.1.32.1	Test Purpose and Environment	2509

A.9.1.32.2	Test parameters	2510

A.9.1.32.3	Test Requirements	2512

A.9.1.33	FDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	2512

A.9.1.33.1	Test Purpose and Environment	2512

A.9.1.33.2	Test parameters	2512

A.9.1.33.3	Test Requirements	2514

A.9.1.34	TDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	2514

A.9.1.34.1	Test Purpose and Environment	2514

A.9.1.34.2	Test parameters	2514

A.9.1.34.3	Test Requirements	2516

A.9.1.35	FDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	2516

A.9.1.35.1	Test Purpose and Environment	2516

A.9.1.35.2	Test parameters	2516

A.9.1.35.3	Test Requirements	2518

A.9.1.36	TDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	2519

A.9.1.36.1	Test Purpose and Environment	2519

A.9.1.36.2	Test parameters	2519

A.9.1.36.3	Test Requirements	2521

A.9.1.37	3 DL PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	2521

A.9.1.37.1	Test Purpose and Environment	2521

A.9.1.37.2	Test parameters	2521

A.9.1.37.3	Test Requirements	2523

A.9.1.38	3 DL PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	2524

A.9.1.38.1	Test Purpose and Environment	2524

A.9.1.38.2	Test parameters	2524

A.9.1.38.3	Test Requirements	2526

A.9.1.39	3 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2527

A.9.1.39.1	Test Purpose and Environment	2527

A.9.1.39.2	Test parameters	2527

A.9.1.39.3	Test Requirements	2531

A.9.1.40	3 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2531

A.9.1.40.1	Test Purpose and Environment	2531

A.9.1.40.2	Test parameters	2531

A.9.1.40.3	Test Requirements	2536

A.9.1.41	FD-FDD RSRP Intra frequency case for UE category 0	2536

A.9.1.41.1	Test Purpose and Environment	2536

A.9.1.41.2	Test parameters	2536

A.9.1.41.3	Test Requirements	2538

A.9.1.42	HD-FDD RSRP Intra frequency case for UE category 0	2538

A.9.1.42.1	Test Purpose and Environment	2538

A.9.1.42.2	Test parameters	2538

A.9.1.42.3	Test Requirements	2540

A.9.1.43	TDD RSRP Intra frequency case for UE category 0	2540

A.9.1.43.1	Test Purpose and Environment	2540

A.9.1.43.2	Test parameters	2540

A.9.1.43.3	Test Requirements	2541

A.9.1.44	4 DL CA PCell in FDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	2542

A.9.1.44.1	Test Purpose and Environment	2542

A.9.1.44.2	Test parameters	2542

A.9.1.44.3	Test Requirements	2546

A.9.1.45	4 DL CA PCell in TDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	2546

A.9.1.45.1	Test Purpose and Environment	2546

A.9.1.45.2	Test parameters	2546

A.9.1.45.3	Test Requirements	2550

A.9.1.46	4 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2550

A.9.1.46.1	Test Purpose and Environment	2550

A.9.1.46.2	Test parameters	2550

A.9.1.46.3	Test Requirements	2556

A.9.1.47	4 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2556

A.9.1.47.1	Test Purpose and Environment	2556

A.9.1.47.2	Test parameters	2557

A.9.1.47.3	Test Requirements	2563

A.9.1.48	5 DL FDD-TDD with PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	2563

A.9.1.48.1	Test Purpose and Environment	2563

A.9.1.48.2	Test parameters	2564

A.9.1.48.3	Test Requirements	2571

A.9.1.49	5 DL FDD-TDD with PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	2572

A.9.1.49.1	Test Purpose and Environment	2572

A.9.1.49.2	Test parameters	2572

A.9.1.49.3	Test Requirements	2579

A.9.1.50	5 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2580

A.9.1.50.1	Test Purpose and Environment	2580

A.9.1.50.2	Test parameters	2580

A.9.1.50.3	Test Requirements	2585

A.9.1.51	5 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2585

A.9.1.51.1	Test Purpose and Environment	2585

A.9.1.51.2	Test parameters	2585

A.9.1.51.3	Test Requirements	2589

A.9.1.52	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2590

A.9.1.52.1	Test Purpose and Environment	2590

A.9.1.52.2	Test parameters	2590

A.9.1.52.3	Test Requirements	2592

A.9.1.52A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2592

A.9.1.52A.1	Test Purpose and Environment	2592

A.9.1.52A.2	Test parameters	2592

A.9.1.52A.3	Test Requirements	2593

A.9.1.53	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2593

A.9.1.53.1	Test Purpose and Environment	2593

A.9.1.53.2	Test parameters	2594

A.9.1.53.3	Test Requirements	2596

A.9.1.53A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2596

A.9.1.53A.1	Test Purpose and Environment	2596

A.9.1.53A.2	Test parameters	2596

A.9.1.53A.3	Test Requirements	2597

A.9.1.54	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2597

A.9.1.54.1	Test Purpose and Environment	2597

A.9.1.54.2	Test parameters	2598

A.9.1.54.3	Test Requirements	2600

A.9.1.55	FS3 Intra frequency absolute and relative RSRP accuracies with FDD PCell	2600

A.9.1.55.1	Test Purpose and Environment	2600

A.9.1.55.2	Test parameters	2600

A.9.1.55.3	Test Requirements	2603

A.9.1.56	FS3 Intra frequency absolute and relative RSRP accuracies with TDD PCell	2603

A.9.1.56.1	Test Purpose and Environment	2603

A.9.1.56.2	Test parameters	2603

A.9.1.56.3	Test Requirements	2605

A.9.1.57	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2606

A.9.1.57.1	Test Purpose and Environment	2606

A.9.1.57.2	Test parameters	2606

A.9.1.57.3	Test Requirements	2608

A.9.1.57A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2608

A.9.1.57A.1	Test Purpose and Environment	2608

A.9.1.57A.2	Test parameters	2608

A.9.1.57A.3	Test Requirements	2609

A.9.1.58	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2609

A.9.1.58.1	Test Purpose and Environment	2609

A.9.1.58.2	Test parameters	2610

A.9.1.58.3	Test Requirements	2612

A.9.1.58A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2612

A.9.1.58A.1	Test Purpose and Environment	2612

A.9.1.58A.2	Test parameters	2612

A.9.1.58A.3	Test Requirements	2613

A.9.1.59	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2613

A.9.1.59.1	Test Purpose and Environment	2613

A.9.1.59.2	Test parameters	2614

A.9.1.59.3	Test Requirements	2616

A.9.1.60	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with FDD PCell	2616

A.9.1.60.1	Test Purpose and Environment	2616

A.9.1.60.2	Test parameters	2616

A.9.1.60.3	Test Requirements	2618

A.9.1.61	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with TDD PCell	2619

A.9.1.61.1	Test Purpose and Environment	2619

A.9.1.61.2	Test parameters	2619

A.9.1.61.3	Test Requirements	2621

A.9.1.62	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2621

A.9.1.62.1	Test Purpose and Environment	2621

A.9.1.62.2	Test parameters	2621

A.9.1.62.3	Test Requirements	2623

A.9.1.63	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2623

A.9.1.63.1	Test Purpose and Environment	2623

A.9.1.63.2	Test parameters	2623

A.9.1.63.3	Test Requirements	2625

A.9.1.64	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2625

A.9.1.64.1	Test Purpose and Environment	2625

A.9.1.64.2	Test parameters	2625

A.9.1.64.3	Test Requirements	2626

A.9.1.65	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2627

A.9.1.65.1	Test Purpose and Environment	2627

A.9.1.65.2	Test parameters	2627

A.9.1.66	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2631

A.9.1.66.1	Test Purpose and Environment	2631

A.9.1.66.2	Test parameters	2631

A.9.1.66.3	Test Requirements	2635

A.9.1.67	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2635

A.9.1.67.1	Test Purpose and Environment	2635

A.9.1.67.2	Test parameters	2635

A.9.1.67.3	Test Requirements	2636

A.9.2	RSRQ	2637

A.9.2.1	FDD Intra frequency case	2637

A.9.2.1.1	Test Purpose and Environment	2637

A.9.2.1.2	Test parameters	2637

A.9.2.1.3	Test Requirements	2639

A.9.2.2	TDD Intra frequency case	2639

A.9.2.2.1	Test Purpose and Environment	2639

A.9.2.2.2	Test parameters	2639

A.9.2.2.3	Test Requirements	2641

A.9.2.3	FDD—FDD Inter frequency case	2641

A.9.2.3.1	Test Purpose and Environment	2641

A.9.2.3.2	Test parameters	2641

A.9.2.3.3	Test Requirements	2643

A.9.2.4	TDD—TDD Inter frequency case	2643

A.9.2.4.1	Test Purpose and Environment	2643

A.9.2.4.2	Test parameters	2643

A.9.2.4.3	Test Requirements	2646

A.9.2.4A	FDD—TDD Inter frequency case	2646

A.9.2.4A.1	Test Purpose and Environment	2646

A.9.2.4A.2	Test parameters	2646

A.9.2.4A.3	Test Requirements	2648

A.9.2.5	FDD RSRQ for E-UTRA Carrier Aggregation	2649

A.9.2.5.1	Test Purpose and Environment	2649

A.9.2.5.2	Test parameters	2649

A.9.2.5.3	Test Requirements	2651

A.9.2.6	TDD RSRQ for E-UTRA Carrier Aggregation	2651

A.9.2.6.1	Test Purpose and Environment	2651

A.9.2.6.2	Test parameters	2651

A.9.2.6.3	Test Requirements	2653

A.9.2.7	FDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2653

A.9.2.7.1	Test Purpose and Environment	2653

A.9.2.7.2	Test parameters	2653

A.9.2.7.3	Test Requirements	2656

A.9.2.8	TDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2656

A.9.2.8.1	Test Purpose and Environment	2656

A.9.2.8.2	Test parameters	2656

A.9.2.8.3	Test Requirements	2659

A.9.2.9	FDD RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS	2659

A.9.2.9.1	Test Purpose and Environment	2659

A.9.2.9.2	Test parameters	2659

A.9.2.9.3	Test Requirements	2662

A.9.2.10	TDD Intra frequency case under time domain measurement resource restriction with MBSFN ABS	2662

A.9.2.10.1	Test Purpose and Environment	2662

A.9.2.10.2	Test parameters	2662

A.9.2.10.3	Test Requirements	2665

A.9.2.11	FDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2665

A.9.2.11.1	Test Purpose and Environment	2665

A.9.2.11.2	Test parameters	2665

A.9.2.11.3	Test Requirements	2666

A.9.2.12	TDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2666

A.9.2.12.1	Test Purpose and Environment	2666

A.9.2.12.2	Test parameters	2666

A.9.2.12.3	Test Requirements	2667

A.9.2.13	Void	2668

A.9.2.13.1	Void	2668

A.9.2.13.2	Void	2668

A.9.2.13.3	Void	2668

A.9.2.14	Void	2668

A.9.2.14.1	Void	2668

A.9.2.14.2	Void	2668

A.9.2.14.3	Void	2668

A.9.2.15	FDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2668

A.9.2.15.1	Test Purpose and Environment	2668

A.9.2.15.2	Test parameters	2668

A.9.2.15.3	Test Requirements	2671

A.9.2.16	TDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2671

A.9.2.16.1	Test Purpose and Environment	2671

A.9.2.16.2	Test parameters	2671

A.9.2.16.3	Test Requirements	2674

A.9.2.17	FDD Intra frequency case for 5 MHz bandwidth	2674

A.9.2.17.1	Test Purpose and Environment	2674

A.9.2.17.2	Test parameters	2674

A.9.2.17.3	Test Requirements	2676

A.9.2.18	FDD—FDD Inter frequency case for 5MHz bandwidth	2676

A.9.2.18.1	Test Purpose and Environment	2676

A.9.2.18.2	Test parameters	2676

A.9.2.18.3	Test Requirements	2678

A.9.2.19	FDD-FDD Inter Frequency WB-RSRQ	2678

A.9.2.19.1	Test Purpose and Environment	2678

A.9.2.19.2	Test parameters	2678

A.9.2.19.3	Test Requirements	2680

A.9.2.20	TDD—TDD Inter Frequency WB-RSRQ	2680

A.9.2.20.1	Test Purpose and Environment	2680

A.9.2.20.2	Test parameters	2680

A.9.2.20.3	Test Requirements	2682

A.9.2.21	FDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2682

A.9.2.21.1	Test Purpose and Environment	2682

A.9.2.21.2	Test parameters	2682

A.9.2.21.3	Test Requirements	2684

A.9.2.22	TDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2684

A.9.2.22.1	Test Purpose and Environment	2684

A.9.2.22.2	Test parameters	2684

A.9.2.22.3	Test Requirements	2685

A.9.2.23	FDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2685

A.9.2.23.1	Test Purpose and Environment	2685

A.9.2.23.2	Test parameters	2685

A.9.2.23.3	Test Requirements	2687

A.9.2.24	TDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2687

A.9.2.24.1	Test Purpose and Environment	2687

A.9.2.24.2	Test parameters	2687

A.9.2.24.3	Test Requirements	2687

A.9.2.25	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	2688

A.9.2.25.1	Test Purpose and Environment	2688

A.9.2.25.2	Test parameters	2688

A.9.2.25.3	Test Requirements	2691

A.9.2.26	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	2691

A.9.2.26.1	Test Purpose and Environment	2691

A.9.2.26.2	Test parameters	2691

A.9.2.26.3	Test Requirements	2694

A.9.2.27	TDD RSRQ for E-UTRAN Carrier Aggregation for 20MHz+10MHz	2694

A.9.2.27.1	Test Purpose and Environment	2694

A.9.2.27.2	Test parameters	2694

A.9.2.27.3	Test Requirements	2695

A.9.2.28	FDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2695

A.9.2.28.1	Test Purpose and Environment	2695

A.9.2.28.2	Test parameters	2695

A.9.2.28.3	Test Requirements	2697

A.9.2.29	TDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2697

A.9.2.29.1	Test Purpose and Environment	2697

A.9.2.29.2	Test parameters	2697

A.9.2.29.3	Test Requirements	2699

A.9.2.30	FDD-FDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2699

A.9.2.30.1	Test Purpose and Environment	2699

A.9.2.30.2	Test parameters	2699

A.9.2.30.3	Test Requirements	2701

A.9.2.31	TDD-TDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2701

A.9.2.31.1	Test Purpose and Environment	2701

A.9.2.31.2	Test parameters	2701

A.9.2.31.3	Test Requirements	2703

A.9.2.32	FDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2703

A.9.2.32.1	Test Purpose and Environment	2703

A.9.2.32.2	Test parameters	2703

A.9.2.32.3	Test Requirements	2705

A.9.2.33	TDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2705

A.9.2.33.1	Test Purpose and Environment	2705

A.9.2.33.2	Test parameters	2705

A.9.2.33.3	Test Requirements	2707

A.9.2.34	FDD—FDD Inter frequency new RSRQ	2707

A.9.2.34.1	Test Purpose and Environment	2707

A.9.2.34.2	Test parameters	2707

A.9.2.34.3	Test Requirements	2709

A.9.2.35	TDD—TDD Inter frequency new RSRQ	2709

A.9.2.35.1	Test Purpose and Environment	2709

A.9.2.35.2	Test parameters	2709

A.9.2.35.3	Test Requirements	2711

A.9.2.36	FDD—FDD Inter frequency RSRQ measured on all OFDM symbols	2711

A.9.2.36.1	Test Purpose and Environment	2711

A.9.2.36.2	Test parameters	2711

A.9.2.36.3	Test Requirements	2712

A.9.2.37	TDD—TDD Inter frequency RSRQ measurement on all OFDM symbols	2713

A.9.2.37.1	Test Purpose and Environment	2713

A.9.2.37.2	Test parameters	2713

A.9.2.37.3	Test Requirements	2714

A.9.2.38	3 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2715

A.9.2.38.1	Test Purpose and Environment	2715

A.9.2.38.2	Test parameters	2715

A.9.2.38.3	Test Requirements	2718

A.9.2.39	3 DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2718

A.9.2.39.1	Test Purpose and Environment	2718

A.9.2.39.2	Test parameters	2718

A.9.2.39.3	Test Requirements	2721

A.9.2.40	3 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2721

A.9.2.40.1	Test Purpose and Environment	2721

A.9.2.40.2	Test parameters	2721

A.9.2.40.3	Test Requirements	2723

A.9.2.41	3 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2723

A.9.2.41.1	Test Purpose and Environment	2723

A.9.2.41.2	Test parameters	2724

A.9.2.41.3	Test Requirements	2726

A.9.2.42	FD-FDD RSRQ Intra frequency case for UE category 0	2726

A.9.2.42.1	Test Purpose and Environment	2726

A.9.2.42.2	Test parameters	2726

A.9.2.42.3	Test Requirements	2728

A.9.2.43	HD-FDD RSRQ Intra frequency case for UE category 0	2728

A.9.2.43.1	Test Purpose and Environment	2728

A.9.2.43.2	Test parameters	2728

A.9.2.43.3	Test Requirements	2730

A.9.2.44	TDD RSRQ Intra frequency case for UE category 0	2730

A.9.2.44.1	Test Purpose and Environment	2730

A.9.2.44.2	Test parameters	2730

A.9.2.44.3	Test Requirements	2732

A.9.2.45	4 DL CA PCell in FDD FDD-TDD RSRQ for E-UTRAN in Carrier Aggregation	2732

A.9.2.45.1	Test Purpose and Environment	2732

A.9.2.45.2	Test parameters	2732

A.9.2.45.3	Test Requirements	2735

A.9.2.46	4 DL CA PCell in TDD TDD-FDD RSRQ for E-UTRAN in Carrier Aggregation	2735

A.9.2.46.1	Test Purpose and Environment	2736

A.9.2.46.2	Test parameters	2736

A.9.2.46.3	Test Requirements	2739

A.9.2.47	5 DL FDD-TDD with PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2739

A.9.2.47.1	Test Purpose and Environment	2740

A.9.2.47.2	Test parameters	2740

A.9.2.47.3	Test Requirements	2743

A.9.2.48	5 DL FDD-TDD with PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2743

A.9.2.48.1	Test Purpose and Environment	2743

A.9.2.48.2	Test parameters	2743

A.9.2.48.3	Test Requirements	2746

A.9.2.49	5 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2746

A.9.2.49.1	Test Purpose and Environment	2746

A.9.2.49.2	Test parameters	2746

A.9.2.49.3	Test Requirements	2750

A.9.2.50	5 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2751

A.9.2.50.1	Test Purpose and Environment	2751

A.9.2.50.2	Test parameters	2751

A.9.2.50.3	Test Requirements	2755

A.9.2.51	FS3 Intra frequency absolute and relative RSRQ accuracies with FDD PCell	2755

A.9.2.51.1	Test Purpose and Environment	2755

A.9.2.51.2	Test parameters	2756

A.9.2.51.3	Test Requirements	2759

A.9.2.52	FS3 Intra frequency absolute and relative RSRQ accuracies with TDD PCell	2759

A.9.2.52.1	Test Purpose and Environment	2759

A.9.2.52.2	Test parameters	2759

A.9.2.52.3	Test Requirements	2761

A.9.2.53	4DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2762

A.9.2.53.1	Test Purpose and Environment	2762

A.9.2.53.2	Test parameters	2762

A.9.2.53.3	Test Requirements	2766

A.9.2.54	4DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2767

A.9.2.54.1	Test Purpose and Environment	2767

A.9.2.54.2	Test parameters	2767

A.9.2.54.3	Test Requirements	2771

A.9.3	UTRAN FDD CPICH RSCP	2771

A.9.3.1	E-UTRAN FDD	2771

A.9.3.1.1	Test Purpose and Environment	2771

A.9.3.1.2	Parameters	2771

A.9.3.1.3	Test Requirements	2774

A.9.3.2 E-UTRAN TDD	2774

A.9.3.2.1	Test Purpose and Environment	2774

A.9.3.2.2	Parameters	2774

A.9.3.2.3	Test Requirements	2777

A.9.3.3	E-UTRAN FDD for 5MHz Bandwidth	2777

A.9.3.3.1	Test Purpose and Environment	2777

A.9.3.3.2	Parameters	2777

A.9.3.3.3	Test Requirements	2778

A.9.4	UTRAN FDD CPICH Ec/No	2779

A.9.4.1	E-UTRAN FDD	2779

A.9.4.1.1	Test Purpose and Environment	2779

A.9.4.1.2	Parameters	2779

A.9.4.1.3	Test Requirements	2781

A.9.4.2	E-UTRAN TDD	2782

A.9.4.2.1	Test Purpose and Environment	2782

A.9.4.2.2	Parameters	2782

A.9.4.2.3	Test Requirements	2784

A.9.4.3	E-UTRAN FDD for 5MHz Bandwidth	2785

A.9.4.3.1	Test Purpose and Environment	2785

A.9.4.3.2	Parameters	2785

A.9.4.3.3	Test Requirements	2786

A.9.5	UTRAN TDD measurement	2786

A.9.5.1	P-CCPCH RSCP absolute accuracy for E-UTRAN FDD	2786

A.9.5.1.1	Test Purpose and Environment	2786

A.9.5.1.2	Test parameters	2786

A.9.5.1.3	Test Requirements	2788

A.9.5.2	P-CCPCH RSCP absolute accuracy for E-UTRAN TDD	2789

A.9.5.2.1	Test Purpose and Environment	2789

A.9.5.2.2	Test parameters	2789

A.9.5.2.3	Test Requirements	2790

A.9.6	GSM Carrier RSSI	2791

A.9.6.1	E-UTRAN FDD	2791

A.9.6.1.1	Test Purpose and Environment	2791

A.9.6.1.2	Test Requirements	2792

A.9.6.2	E-UTRAN TDD	2792

A.9.6.2.1	Test Purpose and Environment	2792

A.9.6.2.2	Test Requirements	2795

A.9.7	UE Rx – Tx Time Difference	2795

A.9.7.1	E-UTRAN FDD UE Rx – Tx time difference case	2795

A.9.7.1.1	Test Purpose and Environment	2795

A.9.7.1.2	Test parameters	2795

A.9.7.1.3	Test Requirements	2797

A.9.7.2	E-UTRA TDD	2797

A.9.7.2.1	Test Purpose and Environment	2797

A.9.7.2.2	Test parameters	2797

A.9.7.2.3	Test Requirements	2799

A.9.7.3	E-UTRAN FDD UE Rx–Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2799

A.9.7.3.1	Test Purpose and Environment	2799

A.9.7.3.2	Test parameters	2799

A.9.7.3.3	Test Requirements	2802

A.9.7.4	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2802

A.9.7.4.1	Test Purpose and Environment	2802

A.9.7.4.2	Test Parameters	2802

A.9.7.4.3	Test Requirements	2805

A.9.7.5	E-UTRAN FDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2805

A.9.7.5.1	Test Purpose and Environment	2805

A.9.7.5.2	Test parameters	2805

A.9.7.5.3	Test Requirements	2808

A.9.7.6	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2808

A.9.7.6.1	Test Purpose and Environment	2808

A.9.7.6.2	Test Parameters	2808

A.9.7.6.3	Test Requirements	2811

A.9.7.7	E-UTRAN FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2811

A.9.7.7.1	Test Purpose and Environment	2811

A.9.7.7.2	Test parameters	2811

A.9.7.7.3	Test Requirements	2812

A.9.7.8	E-UTRAN HD-FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2813

A.9.7.8.1	Test Purpose and Environment	2813

A.9.7.8.2	Test parameters	2813

A.9.7.8.3	Test Requirements	2814

A.9.7.9	E-UTRAN TDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2814

A.9.7.9.1	Test Purpose and Environment	2814

A.9.7.9.2	Test parameters	2814

A.9.7.9.3	Test Requirements	2815

A.9.8	RSTD	2816

A.9.8.1	E-UTRAN FDD RSTD intra frequency case	2816

A.9.8.1.1	Test Purpose and Environment	2816

A.9.8.1.2	Test Requirements	2818

A.9.8.1.2A	Test Requirements for UE Category 1bis	2818

A.9.8.2	E-UTRAN TDD RSTD intra frequency case	2819

A.9.8.2.1	Test Purpose and Environment	2819

A.9.8.2.2	Test Requirements	2821

A.9.8.2.2A	Test Requirements for UE Category 1bis	2821

A.9.8.3	E-UTRAN FDD-FDD RSTD inter frequency case	2822

A.9.8.3.1	Test Purpose and Environment	2822

A.9.8.3.2	Test Requirements	2824

A.9.8.3.2A	Test Requirements for UE Category 1bis	2824

A.9.8.4	E-UTRAN TDD-TDD RSTD inter frequency case	2825

A.9.8.4.1	Test Purpose and Environment	2825

A.9.8.4.2	Test Requirements	2827

A.9.8.4.2A	Test Requirements for UE Category 1bis	2827

A.9.8.5	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation	2828

A.9.8.5.1	Test Purpose and Environment	2828

A.9.8.5.2	Test Requirements	2830

A.9.8.6	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation	2830

A.9.8.6.1	Test Purpose and Environment	2830

A.9.8.6.2	Test Requirements	2833

A.9.8.7	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2833

A.9.8.7.1	Test Purpose and Environment	2833

A.9.8.7.2	Test Requirements	2834

A.9.8.8	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2834

A.9.8.8.1	Test Purpose and Environment	2834

A.9.8.8.2	Test Requirements	2835

A.9.8.9	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2835

A.9.8.9.1	Test Purpose and Environment	2835

A.9.8.9.2	Test Requirements	2836

A.9.8.10	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2836

A.9.8.10.1	Test Purpose and Environment	2836

A.9.8.10.2	Test Requirements	2836

A.9.8.11	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 5 + 5MHz bandwidth	2836

A.9.8.11.1	Test Purpose and Environment	2836

A.9.8.11.2	Test Requirements	2837

A.9.8.12	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 5+5MHz bandwidth	2837

A.9.8.12.1	Test Purpose and Environment	2837

A.9.8.12.2	Test Requirements	2838

A.9.8.13	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz+10MHz	2838

A.9.8.13.1	Test Purpose and Environment	2838

A.9.8.13.2	Test Requirements	2839

A.9.8.14	E-UTRAN FDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2839

A.9.8.14.1	Test Purpose and Environment	2839

A.9.8.14.2	Test Requirements	2844

A.9.8.15	E-UTRAN TDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2844

A.9.8.15.1	Test Purpose and Environment	2844

A.9.8.15.2	Test Requirements	2848

A.9.8.16	HD – FDD Intra frequency case for UE Category NB1 inband mode in normal coverage	2848

A.9.8.16.1	Test Purpose and Environment	2848

A.9.8.16.2	Test Requirements	2851

A.9.8.17	HD – FDD Inter frequency case for UE Category NB1 inband mode in normal coverage	2851

A.9.8.17.1	Test Purpose and Environment	2851

A.9.8.17.2	Test Requirements	2854

A.9.8.18	HD – FDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	2854

A.9.8.18.1	Test Purpose and Environment	2854

A.9.8.18.2	Test Requirements	2857

A.9.8.19	HD – FDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	2857

A.9.8.19.1	Test Purpose and Environment	2857

A.9.8.19.2	Test Requirements	2860

A.9.8.20	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode A	2860

A.9.8.20.1	Test Purpose and Environment	2860

A.9.8.20.2	Test Requirements	2862

A.9.8.21	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CEModeA	2862

A.9.8.21.1	Test Purpose and Environment	2862

A.9.8.21.2	Test Requirements	2865

A.9.8.22	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode A	2865

A.9.8.22.1	Test Purpose and Environment	2865

A.9.8.22.2	Test Requirements	2868

A.9.8.23	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode B	2869

A.9.8.23.1	Test Purpose and Environment	2869

A.9.8.23.2	Test Requirements	2871

A.9.8.24	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CE Mode B	2871

A.9.8.24.1	Test Purpose and Environment	2871

A.9.8.24.2	Test Requirements	2874

A.9.8.25	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode B	2874

A.9.8.25.1	Test Purpose and Environment	2874

A.9.8.25.2	Test Requirements	2877

A.9.8.26	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	2878

A.9.8.26.1	Test Purpose and Environment	2878

A.9.8.26.2	Test Requirements	2880

A.9.8.27	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	2880

A.9.8.27.1	Test Purpose and Environment	2880

A.9.8.27.2	Test Requirements	2883

A.9.8.28	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode A	2883

A.9.8.28.1	Test Purpose and Environment	2883

A.9.8.28.2	Test Requirements	2886

A.9.8.29	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	2886

A.9.8.29.1	Test Purpose and Environment	2886

A.9.8.29.2	Test Requirements	2889

A.9.8.30	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	2890

A.9.8.30.1	Test Purpose and Environment	2890

A.9.8.30.2	Test Requirements	2892

A.9.8.31	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode B	2893

A.9.8.31.1	Test Purpose and Environment	2893

A.9.8.31.2	Test Requirements	2896

A.9.9	RSRP and RSRQ on the serving cell	2896

A.9.9.1	FDD Intra frequency serving cell case	2896

A.9.9.1.1	Test Purpose and Environment	2896

A.9.9.1.2	Test parameters	2896

A.9.9.1.3	Test Requirements	2898

A.9.9.2	TDD Intra frequency serving cell case	2898

A.9.9.2.1	Test Purpose and Environment	2898

A.9.9.2.2	Test parameters	2898

A.9.9.2.3	Test Requirements	2900

A.9.10	SSTD	2900

A.9.10.1	EUTRAN FDD-FDD SSTD accuracy in asynchronous DC	2900

A.9.10.1.1	Test Purpose and Environment	2900

A.9.10.1.2	Test parameters	2900

A.9.10.1.3	Test Requirements	2902

A.9.10.2	Void	2902

A.9.10.3	Void	2902

A.9.10.4	Void	2902

A.9.11	RSSI	2902

A.9.11.1	FS3 average RSSI accuracy case (PCell using FDD)	2902

A.9.11.1.1	Test Purpose and Environment	2902

A.9.11.1.2	Test parameters	2902

A.9.11.1.3	Test Requirements	2904

A.9.11.2	FS3 average RSSI accuracy case (PCell using TDD)	2904

A.9.11.2.1	Test Purpose and Environment	2904

A.9.11.2.2	Test parameters	2904

A.9.12	Channel occupancy	2906

A.9.12.1	FS3 channel occupancy test (PCell using FDD)	2906

A.9.12.1.1	Test Purpose and Environment	2906

A.9.12.1.2	Test parameters	2906

A.9.12.1.3	Test Requirements	2908

A.9.12.2	FS3 channel occupancy test (PCell using TDD)	2908

A.9.12.2.1	Test Purpose and Environment	2908

A.9.12.2.2	Test parameters	2908

A.9.12.2.3	Test Requirements	2910

A.9.13	RS-SINR	2910

A.9.13.1	FDD Intra-Frequency Case	2910

A.9.13.1.1	Test Purpose and Environment	2910

A.9.13.1.2	Test parameters	2910

A.9.13.1.3	Test Requirements	2912

A.9.13.2	TDD Intra-Frequency Case	2912

A.9.13.2.1	Test Purpose and Environment	2912

A.9.13.2.2	Test parameters	2912

A.9.13.2.3	Test Requirements	2915

A.9.13.3	FDD—FDD Inter frequency case	2915

A.9.13.3.1	Test Purpose and Environment	2915

A.9.13.3.2	Test parameters	2915

A.9.13.3.3	Test Requirements	2918

A.9.13.4	TDD—TDD Inter frequency case	2918

A.9.13.4.1	Test Purpose and Environment	2918

A.9.13.4.2	Test parameters	2918

A.9.13.4.3	Test Requirements	2922

A.9.13.5	FDD—TDD Inter frequency case	2923

A.9.13.5.1	Test Purpose and Environment	2923

A.9.13.5.2	Test parameters	2923

A.9.13.5.3	Test Requirements	2927

A.9.13.6	TDD—FDD Inter frequency case	2927

A.9.13.6.1	Test Purpose and Environment	2927

A.9.13.6.2	Test parameters	2927

A.9.13.6.3	Test Requirements	2930

A.9.14	Channel quality reporting accuracy	2930

A.9.14.1	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage	2930

A.9.14.1.1	Test Purpose and Environment	2930

A.9.14.1.2	Test parameters	2930

A.9.14.1.3	Test Requirements	2931

A.9.14.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage	2931

A.9.14.2.1	Test Purpose and Environment	2931

A.9.14.2.2	Test parameters	2931

A.9.14.2.3	Test Requirements	2932

A.10	Proximity-based Services in Any Cell Selection State	2932

A.10.1	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	2932

A.10.1.1	Test Purpose and Environment	2932

A.10.1.2	Test Requirements	2933

A.10.2	E-UTRAN FDD – Initiation/Cease of SLSS Transmission with ProSe Direct Communication	2934

A.10.2.1	Test Purpose and Environment	2934

A.10.2.2	Test Requirements	2935

A.10.3	E-UTRAN FDD – SyncRef UE Selection / Reselection Test	2935

A.10.3.1	Test Purpose and Environment	2935

A.10.3.2	Test Requirements	2937

A.10.4	E-UTRAN FDD – Cell Identification on downlink frequency associated with ProSe frequency (when UE is transmitting for ProSe)	2938

A.10.4.1	Test Purpose and Environment	2938

A.10.4.2	Test Requirements	2940

A.11	V2V Sidelink Communication for V2V Operation on Dedicated V2V Carrier	2940

A.11.1	V2V UE Transmission Timing Accuracy Test	2940

A.11.1.1	Test Purpose and Environment	2940

A.11.1.2	Test requirements	2941

A.11.2	Interruptions due to V2V sidelink communication	2941

A.11.2.1	Test Purpose and Environment	2941

A.11.2.2	Test Requirements	2943

A.12	2943

A.12.1	V2X UE Transmission Timing Accuracy Test	2943

A.12.1.1	V2X UE Transmission Timing Accuracy Test for eNB as Timing Reference	2943

A.12.1.1.1	Test Purpose and Environment	2943

A.12.1.1.2	Test requirements	2944

A.12.1.2	V2X UE Transmission Timing Accuracy Test for SyncRef UE as Timing Reference	2945

A.12.1.2.1	Test Purpose and Environment	2945

A.12.1.2.2	Test Requirements	2945

A.12.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication	2946

A.12.2.1	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for eNB as Timing Reference	2946

A.12.2.1.1	Test Purpose and Environment	2946

A.12.2.1.2	Test Requirements	2947

A.12.2.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for SyncRef UE as Timing Reference	2948

A.12.2.2.1	Test Purpose and Environment	2948

A.12.2.2.2	Test Requirements	2949

A.12.3	V2X Synchronization Reference Selection/Reselection Tests	2949

A.12.3.1	V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority	2949

A.12.3.1.1	Test Purpose and Environment	2949

A.12.3.1.2	Test Requirements	2951

A.12.3.2	V2X Synchronization Reference Selection/Reselection Tests for eNB configured as the highest priority	2952

A.12.3.2.1	Test Purpose and Environment	2952

A.12.3.1.2	Test Requirements	2954

A.12.4	Congestion Control Measurement Test for V2X UE	2954

A.12.4.1	Test Purpose and Environment	2954

A.12.4.2	Test Requirements	2956

A.12.5	Interruptions due to V2X Sidelink Communication	2956

A.12.5.1	Test Purpose and Environment	2956

A.12.5.1	Test Requirements	2958

A.12.6	V2X UE Autonomous Resource Selection/Reselection Measurement Test	2958

A.12.6.1	V2X UE Autonomous Resource Selection/Reselection Tests for PSSCH-RSRP measurements	2958

A.12.6.1.1	Test Purpose and Environment	2958

A.12.6.1.2	Test Requirements	2960

A.12.6.2	V2X UE Autonomous Resource Selection/Reselection Tests for S-RSSI measurements	2960

A.12.6.2.1	Test Purpose and Environment	2960

A.12.6.1.2	Test Requirements	2962

Annex B (normative):	Conditions for RRM requirements applicability for operating bands	2963

B.1	Conditions for E-UTRAN RRC_IDLE state mobility	2963

B.1.1	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection	2963

B.1.2	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection	2963

B.1.3	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category M1	2963

B.1.4	Conditions for measurements of intra-frequency NB-IoT cells for cell re-selection for UE Category NB1	2965

B.1.5	Conditions for measurements of inter-frequency NB-IoT cells for cell re-selection for UE Category NB1	2965

B.1.6	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category 1bis	2966

B.1.7	Conditions for measurements of E-UTRAN cells for cell re-selection for UE Category M2	2966

B.1.7.1 Conditions for measurements of intra-frequence E-UTRAN cells for cell selection	2966

B.1.7.2 Condition for measurements of inter-frequence E-UTRAN cells for cell selection	2967

B.1.8	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection for UE Category M1	2969

B.2	Conditions for UE Measurements Procedures in RRC_CONNECTED State	2969

B.2.1	Conditions for E-UTRAN intra-frequency measurements	2969

B.2.2	Conditions for E-UTRAN intra-frequency measurements with autonomous gaps	2970

B.2.3	Conditions for E-UTRAN inter-frequency measurements	2970

B.2.4	Conditions for E-UTRAN inter-frequency measurements with autonomous gaps	2970

B.2.5	Conditions for E-UTRAN OTDOA intra-frequency RSTD Measurements	2971

B.2.6	Conditions for E-UTRAN OTDOA inter-frequency RSTD Measurements	2971

B.2.7	Conditions for Measurements of the secondary component carrier with deactivated SCell	2971

B.2.8	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction	2972

B.2.9	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction with CRS Assistance Information	2972

B.2.10	Conditions for E-UTRAN intra-frequency discovery signal measurements	2973

B.2.10.1	Conditions for E-UTRAN intra-frequency CRS-based measurements	2973

B.2.10.2	Conditions for E-UTRAN intra-frequency CSI-RS based measurements	2973

B.2.11	Conditions for E-UTRAN inter-frequency discovery signal measurements	2974

B.2.11.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	2974

B.2.11.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	2974

B.2.12	Conditions for E-UTRAN intra-frequency discovery signal measurements under operation with frame structure 3	2975

B.2.13	Conditions for E-UTRAN inter-frequency discovery signal measurements under operation with frame structure 3	2975

B.2.13.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	2975

B.2.13.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	2975

B.2.14	Conditions for E-UTRAN intra-frequency measurements by UE Category M1	2976

B.2.15	Conditions for NB-IoT intra-frequency measurements by UE Category NB1	2977

B.2.16	Conditions for NB-IoT intra-frequency RSTD measurements by UE Category NB1	2977

B.2.17	Conditions for NB-IoT inter-frequency RSTD measurements by UE Category NB1	2978

B.2.18	Conditions for E-UTRAN inter-frequency measurements by UE Category M1	2978

B.2.19	Conditions for E-UTRAN measurements by UE Category M2	2980

B.2.19.1	Conditions for E-UTRAN intra-frequency measurements	2980

B.2.19.2	Conditions for E-UTRAN inter-frequency measurements	2980

B.2.20	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M1	2980

B.2.21	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M2	2981

B.2.22	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M1	2981

B.2.23	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M2	2983

B.3	Conditions for measurements performance requirements for UE	2983

B.3.1	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements	2983

B.3.2	Void	2983

B.3.3	Conditions for inter-frequency RSRP and RSRQ Accuracy Requirements	2983

B.3.4	Conditions for inter-frequency relative RSRP and RSRQ Accuracy Requirements	2983

B.3.5	Conditions for UE Rx – Tx time difference	2983

B.3.6	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements	2984

B.3.7	Conditions for inter-frequency RSTD measurements	2984

B.3.8	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements	2984

B.3.9	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction	2984

B.3.10	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction	2984

B.3.11	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	2985

B.3.12	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	2985

B.3.13	Conditions for UE Rx–Tx Time Difference Measurement under Time Domain Measurement Resource Restriction with CRS Assistance Information	2985

B.3.14	Conditions for Intra-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	2985

B.3.14.1	Conditions for Intra-frequency CRS-based measurements	2985

B.3.14.2	Conditions for Intra-frequency CSI-RS-based measurements	2985

B.3.15	Conditions for Intra-Frequency Relative Discovery Signal Measurement Accuracy Requirements	2986

B.3.15.1	Conditions for Intra-frequency CRS-based measurements	2986

B.3.15.2	Conditions for Intra-frequency CSI-RS-based measurements	2986

B.3.16	Conditions for Inter-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	2987

B.3.16.1	Conditions for Inter-frequency CRS-based measurements	2987

B.3.16.2	Conditions for Inter-frequency CSI-RS-based measurements	2987

B.3.17	Conditions for Inter-Frequency Relative Discovery Signal Measurement Accuracy Requirements	2987

B.3.17.1	Conditions for Inter-frequency CRS-based measurements	2987

B.3.17.2	Conditions for Inter-frequency CSI-RS-based measurements	2987

B.3.18	Conditions for Intra-frequency Absolute RS-SINR Accuracy Requirements	2987

B.3.19	Conditions for Inter-frequency Absolute RS-SINR Accuracy Requirements	2987

B.3.20	Conditions for Inter-frequency Relative RS-SINR Accuracy Requirements	2987

B.3.21	Conditions for Intra-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	2988

B.3.21.1	Conditions for RSRP measurements	2988

B.3.21.2	Conditions for RSRQ measurements	2988

B.3.21.3	Conditions for CSI-RSRP measurements	2988

B.3.22	Conditions for Intra-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	2988

B.3.22.1	Conditions for RSRP measurements	2988

B.3.22.2	Void	2989

B.3.22.3	Conditions for CSI-RSRP measurements	2989

B.3.23	Conditions for Inter-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	2989

B.3.23.1	Conditions for RSRP measurements	2989

B.3.23.2	Conditions for RSRQ measurements	2989

B.3.23.3	Conditions for CSI-RSRP measurements	2989

B.3.24	Conditions for Inter-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	2990

B.3.24.1	Conditions for RSRP measurements	2990

B.3.24.2	Conditions for RSRQ measurements	2990

B.3.24.3	Conditions for CSI-RSRP measurements	2990

B.3.25	Conditions for NB-IoT intra-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	2990

B.3.26	Conditions for NB-IoT inter-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	2990

B.3.27	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements for Category 0	2990

B.3.28	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements for Category 0	2991

B.3.29	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for NB1	2991

B.3.30	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for NB1	2991

B.3.31	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	2991

B.3.32	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	2991

B.3.33	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	2992

B.3.34	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	2992

B.4	RRM Requirements Exceptions	2992

B.4.1	General	2992

B.4.2	Receiver sensitivity relaxation for UE supporting CA	2992

B.4.3	Receiver sensitivity relaxation for UE configured with CA	2992

B.4.3.1	Inter-band carrier aggregation	2992

B.4.3.2	Intra-band non-contiguous carrier aggregation	2992

B.4.3.3	Inter-band carrier aggregation with operating bands without uplink band	2993

B.5	Conditions for Measurement Performance Requirements for ProSe UE	2993

B.5.1	Conditions for S-RSRP Accuracy Requirements	2993

B.5.2	Conditions for Relative S-RSRP Accuracy Requirements	2993

B.5.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	2994

B.5.4	Conditions for SD-RSRP Accuracy Requirements	2994

B.5.5	Conditions for Relative SD-RSRP Accuracy Requirements	2995

B.6	Conditions for V2X	2995

B.6.1	Test parameters for GNSS signals	2995

B.6.2	Conditions for Absolute S-RSRP Accuracy Requirements	2995

B.6.3	Conditions for Relative S-RSRP Accuracy Requirements	2995

B.6.4	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	2996

B.6.5	Conditions for Absolute PSSCH-RSRP Accuracy Requirements	2996

Annex C (informative):	Change history:	2997