3GPP TS 36.133 V16.2.0 (2019-06)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Evolved Universal Terrestrial Radio Access (E-UTRA);
Requirements for support of radio resource management

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

# Contents

Foreword	87

1	Scope	88

2	References	88

3	Definitions, symbols and abbreviations	90

3.1	Definitions	90

3.2	Symbols	92

3.3	Abbreviations	93

3.4	Test tolerances	95

3.5	Additional notation	96

3.5.1	Groups of bands	96

3.6	General	98

3.6.1	Applicability of requirements in this specification version	98

3.6.1.1	Applicability of requirements for UE capable of network-based CRS interference mitigation	104

3.6.1.2	Applicability of requirements with CRS muting for category M1 UE capable of CRS muting	106

3.6.1.3	Applicability of requirements with CRS muting for category M2 UE capable of CRS muting	107

3.6.2	Applicability of requirements for EN-DC operation	108

3.6.3	Applicability of requirements for NE-DC operation	109

3.6.4	Applicability of requirements for NGEN-DC operation	110

4	E-UTRAN RRC_IDLE state mobility	110

4.1	Cell Selection	110

4.2	Cell Re-selection	110

4.2.1	Introduction	110

4.2.2	Requirements	110

4.2.2.1	Measurement and evaluation of serving cell	111

4.2.2.2	Void	112

4.2.2.3	Measurements of intra-frequency E-UTRAN cells	112

4.2.2.4	Measurements of inter-frequency E-UTRAN cells	114

4.2.2.5	Measurements of inter-RAT cells	116

4.2.2.5.1	Measurements of UTRAN FDD cells	116

4.2.2.5.2	Measurements of UTRAN TDD cells	118

4.2.2.5.3	Measurements of GSM cells	119

4.2.2.5.4	Measurements of HRPD cells	120

4.2.2.5.5	Measurements of cdma2000 1X	121

4.2.2.5.6	Measurements of NR cells	122

4.2.2.6	Evaluation of cell re-selection criteria	123

4.2.2.7	Maximum interruption in paging reception	123

4.2.2.8	void	124

4.2.2.9	UE measurement capability	124

4.2.2.9a	UE measurement capability (Increased UE carrier monitoring)	125

4.2.2.10	Reselection to CSG cells	125

4.2.2.10.1	Reselection from a non CSG to an inter-frequency CSG cell	125

4.2.2.10.2	Reselection from a non CSG to an inter-RAT UTRAN FDD CSG cell	126

4.2.2.11	Void	127

4.2.2.12	Void	127

4.2.2.13	Void	127

4.3	Minimization of Drive Tests (MDT)	127

4.3.1	Introduction	128

4.3.2	Measurements	128

4.3.2.1	Requirements	128

4.3.3	Relative Time Stamp Accuracy	128

4.3.3.1	Requirements	128

4.3.4	Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	128

4.3.4.1	 Requirements	128

4.3.5	Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	129

4.3.5.1	Requirements for timeSinceFailure	129

4.4	MBSFN Measurements	129

4.4.1	Introduction	129

4.4.2	MBSFN RSRP measurements	129

4.4.3	MBSFN RSRQ measurements	129

4.4.4	MCH BLER measurements	130

4.5	Proximity-based Services	130

4.5.1	Introduction	130

4.5.2	Requirements	130

4.5.2.1	Interruptions with ProSe Direct Discovery	130

4.5.2.2	Interruptions with ProSe Direct Communication	130

4.5.2.3	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	130

4.5.2.4	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	131

4.6	Cell Selection and Re-selection Requirements for UE category NB1	131

4.6.1	Cell Selection	131

4.6.2	Cell Re-selection	131

4.6.2.1	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage	132

4.6.2.1A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in normal coverage when configured with WUS	132

4.6.2.2	Measurements of intra-frequency NB-IoT cells for UE category NB1 in normal coverage	133

4.6.2.3	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in enhanced coverage	134

4.6.2.3A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in enhanced coverage when configured with WUS	135

4.6.2.4	Measurements of intra-frequency NB-IoT cells for UE category NB1 in enhanced coverage	136

4.6.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage	138

4.6.2.6	Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage	139

4.6.2.7	Maximum interruption in paging reception in normal coverage	140

4.6.2.7A	Maximum interruption in paging reception in enhanced coverage	140

4.6.2.8	UE measurement capability	140

4.6.2.9	WUS receptions for NB1	141

4.7	Cell Selection and Re-selection Requirements for UE category M1	141

4.7.1	Cell Selection	141

4.7.2	Cell Re-selection	141

4.7.2.1	Cell Re-selection requirements for UE category M1 in normal coverage	141

4.7.2.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	141

4.7.2.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	142

4.7.2.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	143

4.7.2.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	145

4.7.2.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	145

4.7.2.2	Cell Re-selection requirements for UE category M1 in enhanced coverage	145

4.7.2.2.1	Measurement and evaluation of serving cell for UE category M1 in enhanced coverage	145

4.7.2.2.2	Measurements of intra-frequency cells for UE category M1 in enhanced coverage	146

4.7.2.2.3	Measurements of inter-frequency cells for UE category M1 in enhanced coverage	148

4.7.2.2.4	Maximum allowed layers for multiple monitoring for UE category M1 in enhanced coverage	149

4.7.2.2.5	Maximum interruption in paging reception for Category M1 UEs in extended coverage	150

4.7.2.3	WUS receptions for UE category M1	150

4.8	Idle State Positioning Measurement Requirements for UE category NB1	150

4.8.1	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for normal coverage	150

4.8.1.1	RSTD Measurement Reporting Delay	152

4.8.2	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	152

4.8.2.1	RSTD Measurement Reporting Delay	154

4.8.3	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for normal coverage	154

4.8.3.1	RSTD Measurement Reporting Delay	155

4.8.4	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	156

4.8.4.1	RSTD Measurement Reporting Delay	157

4.8.5	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	158

4.8.5.1	Measurement Reporting Delay	159

4.8.6	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	159

4.8.6.1	Measurement Reporting Delay	160

4.8.7	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	160

4.8.7.1	Measurement Reporting Delay	161

4.8.8	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	162

4.8.8.1	Measurement Reporting Delay	163

4.9	Idle Mode CA Measurement	164

4.9.1	Introduction	164

4.9.2	Requirement	164

4.9.2.1	Detected cell requirement during state transition and Idle mode	164

4.9.2.2	Measurements of inter-frequency CA candidate cells	164

4.9.2.3	Measurements on serving cell	165

4A	E-UTRAN RRC_INACTIVE state mobility	165

4A.1	Cell Re-selection	165

4A.1.1	Introduction	165

4A.1.2	Requirements	165

4A.1.2.1	UE measurement capability	165

4A.1.2.2	Measurement and evaluation of serving cell	165

4A.1.2.3	Measurements of intra-frequency E-UTRAN cells	165

4A.1.2.4	Measurements of inter-frequency E-UTRAN cells	165

4A.1.2.5	Evaluation of cell re-selection criteria	166

4A.1.2.6	Maximum interruption in paging reception	166

4A.1.2.7	Measurements of inter-RAT NR cells	166

5	E-UTRAN RRC_CONNECTED state mobility	166

5.1	E-UTRAN Handover	166

5.1.1	Introduction	166

5.1.2	Requirements	166

5.1.2.1	E-UTRAN FDD – FDD	166

5.1.2.1.1	Handover delay	166

5.1.2.1.2	Interruption time	167

5.2.2.2	E-UTRAN FDD – TDD	168

5.2.2.2.1	(Void)	168

5.2.2.2.2	(Void)	168

5.2.2.3	E-UTRAN TDD – FDD	168

5.2.2.3.1	(Void)	168

5.2.2.3.2	(Void)	168

5.2.2.4	E-UTRAN TDD – TDD	168

5.2.2.4.1	Handover delay	168

5.2.2.4.2	Interruption time	169

5.2.2.5	E-UTRAN HD–FDD	170

5.2.2.5.1	Handover delay	170

5.2.2.5.2	Interruption time	171

5.3	Handover to other RATs	172

5.3.1	E-UTRAN - UTRAN FDD Handover	172

5.3.1.1	Introduction	172

5.3.1.1.1	Handover delay	172

5.3.1.1.2	Interruption time	172

5.3.2	E-UTRAN - UTRAN TDD Handover	173

5.3.2.1	Introduction	173

5.3.2.2	Requirements	173

5.3.2.2.1	Handover delay	173

5.3.2.2.2	Interruption time	173

5.3.3	E-UTRAN - GSM Handover	174

5.3.3.1	Introduction	174

5.3.3.2	Requirements	174

5.3.3.2.1	Handover delay	174

5.3.3.2.2	Interruption time	174

5.3.4	E-UTRAN - NR FR1 Handover	175

5.3.4.1	Introduction	175

5.3.4.2	Handover delay	175

5.3.4.3	Interruption time	175

5.3.5	E-UTRAN - NR FR2 Handover	175

5.3.5.1	Introduction	175

5.3.5.2	Handover delay	176

5.3.5.3	Interruption time	176

5.4	Handover to Non-3GPP RATs	176

5.4.1	E-UTRAN – HRPD Handover	176

5.4.1.1	Introduction	176

5.4.1.1.1	Handover delay	176

5.4.1.1.2	Interruption time	177

5.4.2	E-UTRAN – cdma2000 1X Handover	177

5.4.2.1	Introduction	177

5.4.2.1.1	Handover delay	177

5.4.2.1.2	Interruption time	177

5.5	E-UTRAN Handover for Cat-M1 UEs	178

5.5.1	Introduction	178

5.5.2	Requirements in CEModeA	178

5.5.2.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	178

5.5.2.1.1	Handover delay	178

5.5.2.1.2	Interruption time	178

5.5.2.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	179

5.5.2.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	179

5.5.2.3.1	Void	179

5.5.2.3.2	Void	179

5.5.3	Requirements in CEModeB	179

5.5.3.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	179

5.5.3.1.1	Handover delay	179

5.5.3.1.2	Interruption time	179

5.5.3.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	180

5.5.3.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	180

5.6	Void	180

6	RRC Connection Mobility Control	180

6.1	RRC Re-establishment	180

6.1.1	Introduction	180

6.1.2	Requirements	180

6.1.2.1	UE Re-establishment delay requirement	181

6.2	Random Access	181

6.2.1	Introduction	181

6.2.2	Requirements	181

6.2.2.1	Contention based random access	181

6.2.2.1.1	Correct behaviour when receiving Random Access Response reception	181

6.2.2.1.2	Correct behaviour when not receiving Random Access Response reception	182

6.2.2.1.3	Correct behaviour when receiving a NACK on msg3	182

6.2.2.1.4	Void	182

6.2.2.1.5	Correct behaviour when receiving a message over Temporary C-RNTI	182

6.2.2.1.6	Correct behaviour when contention Resolution timer expires	182

6.2.2.2	Non-Contention based random access	182

6.2.2.2.1	Correct behaviour when receiving Random Access Response	182

6.2.2.2.2	Correct behaviour when not receiving Random Access Response	182

6.2.3	Requirements for Cat-M1 UEs	182

6.3	RRC Connection Release with Redirection	183

6.3.1	Introduction	183

6.3.2	Requirements	183

6.3.2.1	RRC connection release with redirection to UTRAN FDD	183

6.3.2.2	RRC connection release with redirection to GERAN	183

6.3.2.3	RRC connection release with redirection to UTRAN TDD	184

6.3.2.4	RRC connection release with redirection to NR	184

6.4	CSG Proximity Indication for E-UTRAN and UTRAN	185

6.4.1	Introduction	185

6.4.2	Requirements	185

6.5	RRC Re-establishment for NB-IoT UEs	185

6.5.1	Introduction	185

6.5.2	Requirements	185

6.5.2.1	UE Re-establishment delay requirement in normal coverage	186

6.5.2.2	UE Re-establishment delay requirement in enhanced coverage	186

6.6	Random Access for UE category NB1	186

6.6.1	Introduction	186

6.6.2	Requirements	187

6.6.2.1	Correct behaviour when receiving Random Access Response reception	187

6.6.2.2	Correct behaviour when not receiving Random Access Response reception	187

6.6.2.3	Correct behaviour when receiving a NACK on msg3	187

6.6.2.4	Correct behaviour when receiving a message over Temporary C-RNTI	187

6.6.2.5	Correct behaviour when contention Resolution timer expires	187

6.6.2.6	MSG3-based channel quality report for UE Category NB1	187

6.6.3	Requirements for NPRACH configuration	188

6.7	RRC Re-establishment for Cat-M1 UEs	188

6.7.1	Introduction	188

6.7.2	Requirements	188

6.7.2.1	UE Re-establishment delay requirement for CEModeA	189

6.7.2.2	UE Re-establishment delay requirement for CEModeB	189

6.8	RRC Connection Release with Redirection for Cat-M1 UEs	190

6.8.1	Introduction	190

6.8.2	Requirements	190

6.8.2.1	RRC connection release with redirection to E-UTRAN with CE Mode A	190

6.9	RRC Connection Redirection to Non-anchor Carrier in NB-IoT	190

6.9.1	Introduction	190

6.9.2	Requirements	191

7	Timing and signalling characteristics	191

7.1	UE transmit timing	191

7.1.1	Introduction	191

7.1.2	Requirements	192

7.2	UE timer accuracy	193

7.2.1	Introduction	193

7.2.2	Requirements	193

7.3	Timing Advance	193

7.3.1	Introduction	193

7.3.2	Requirements	193

7.3.2.1	Timing Advance adjustment delay	193

7.3.2.2	Timing Advance adjustment accuracy	194

7.4	Cell phase synchronization accuracy (TDD)	194

7.4.1	Definition	194

7.4.2	Minimum requirements	194

7.5	Synchronization Requirements for E-UTRAN to 1xRTT and HRPD Handovers	195

7.5.1	Introduction	195

7.5.2	eNodeB Synchronization Requirements	195

7.5.2.1	Synchronized E-UTRAN	195

7.5.2.2	Non-Synchronized E-UTRAN	195

7.6	Radio Link Monitoring	195

7.6.1	Introduction	195

7.6.2	Requirements	197

7.6.2.1	Minimum requirement when no DRX is used	197

7.6.2.2	Minimum requirement when DRX is used	198

7.6.2.3	Minimum requirement at transitions	199

7.6.2.4	Minimum requirement during SI Acquisition with autonomous gaps	199

7.6.2.5	Minimum requirement under IDC Interference	200

7.7	SCell Activation and Deactivation Delay for E-UTRA Carrier Aggregation	200

7.7.1	Introduction	200

7.7.2	SCell Activation Delay Requirement for Deactivated SCell	200

7.7.3	SCell Deactivation Delay Requirement for Activated SCell	201

7.7.4	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	202

7.7.5	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	204

7.7.6	SCell Activation Delay Requirement for Deactivated PUCCH SCell	205

7.7.7	SCell Activation Delay Requirement for Deactivated PUCCH SCell with Multiple SCells	205

7.7.8	SCell Deactivation Delay Requirement for Activated PUCCH SCell	206

7.7.9	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple SCells	206

7.7.10	SCell Activation Delay Requirement for Deactivated SCell under Frame Structure 3	206

7.7.11	SCell Deactivation Delay Requirement for Activated SCell under Frame Structure 3	208

7.7.12	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells under Frame Structure 3	208

7.7.13	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells under Frame Structure 3	209

7.7.14	SCell Activation Delay Requirement for Dormant SCell	210

7.7.15	SCell Hibernation Delay Requirement for Activated SCell	211

7.7.16	SCell Hibernation Delay Requirement for Deactivated SCell	212

7.7.17	SCell Deactivation Delay Requirement for Dormant SCell	213

7.7.18	Direct SCell Activation and Hibernation Delay Requirement	213

7.7.19	Direct SCell Activation and Hibernation Delay Requirement at RRC Reconfiguration during Handover	215

7.8	Interruptions with Carrier Aggregation	217

7.8.1	Introduction	217

7.8.2	Requirements	218

7.8.2.1	Interruptions at SCell addition/release for intra-band CA	218

7.8.2.2	Interruptions at SCell addition/release for inter-band CA	218

7.8.2.3	Interruptions at SCell activation/deactivation for intra-band CA	218

7.8.2.4	Interruptions at SCell activation/deactivation for inter-band CA	218

7.8.2.5	Interruptions during measurements on SCC for intra-band CA	218

7.8.2.6	Interruptions during measurements on SCC for inter-band CA	218

7.8.2.7	Interruptions at SCell addition/release with multiple downlink SCells	219

7.8.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	219

7.8.2.9	Interruptions during measurements on SCC with multiple downlink SCells	219

7.8.2.10	Interruptions at overlapping addition/release/activation/deactivation of SCells	220

7.8.2.11	Interruptions during RSSI measurements on one SCC under Frame Structure 3	220

7.8.2.12	Interruptions during RSSI measurements on multiple SCCs under Frame Structure 3	221

7.8.2.13	Interruptions at SRS carrier based switching	221

7.8.2.14	Interruptions at SCell activation of dormant SCell for intra-band CA	222

7.8.2.15	Interruptions at SCell activation of dormant SCell for inter-band CA	222

7.8.2.16	Interruptions at SCell activation of multiple dormant SCells	222

7.8.2.17	Interruptions during CQI measurement on dormant SCell	223

7.9	Maximum Transmission Timing Difference in Carrier Aggregation	223

7.9.1	Introduction	223

7.9.2	Minimum Requirements for Interband Carrier Aggregation	223

7.9.3	Minimum Requirements for Intraband non-contiguous Carrier Aggregation	223

7.9.4	Minimum Requirements for Inter-Band Carrier Aggregation under Frame Structure 3	224

7.10	Interruptions with RSTD Measurements with Carrier Aggregation	224

7.10.1	Introduction	224

7.10.2	Requirements	224

7.10.2.1	Interruptions during RSTD measurements on SCC for intra-band CA with one downlink SCell	224

7.10.2.2	Interruptions during RSTD measurements on SCC for inter-band CA with one downlink SCell	224

7.10.2.3	Interruptions during RSTD measurements on SCC with multiple downlink SCells	225

7.10.2.4	Interruptions at overlapping RSTD and inter-frequency measurements	225

7.11	Radio Link Monitoring for UE Category 0	225

7.11.1	Introduction	225

7.11.2	Requirements for FD-FDD and TDD	227

7.11.2.1	Minimum requirement when no DRX is used	227

7.11.2.2	Minimum requirement when DRX is used	227

7.11.2.3	Minimum requirement at transitions	228

7.11.3	Requirements for HD-FDD	228

7.11.3.1	Minimum requirement when no DRX is used	228

7.11.3.2	Minimum requirement when DRX is used	228

7.11.3.3	Minimum requirement at transitions	229

7.12	Interruptions with Dual Connectivity	229

7.12.1	Introduction	229

7.12.2	Requirements	229

7.12.2.1	Interruptions at PSCell addition/release	229

7.12.2.2	Interruptions at transitions between active and non-active during DRX	230

7.12.2.3	Interruptions at transitions from non-DRX to DRX	230

7.12.2.4	Interruptions at SCell addition/release	230

7.12.2.5	Interruptions at SCell activation/deactivation	230

7.12.2.6	Interruptions during measurements on SCC	231

7.12.2.7	Interruptions at SRS carrier based switching	231

7.13	Cell phase synchronization accuracy (Synchronized mode of dual connectivity)	232

7.13.1	Definition	232

7.13.2	Minimum requirements	232

7.14	PSCell Addition and Release Delay for E-UTRA Dual Connectivity	232

7.14.1	Introduction	232

7.14.2	PSCell Addition Delay Requirement	232

7.14.3	PSCell Release Delay Requirement	233

7.15	Maximum Receive Timing Difference in Dual Connectivity	233

7.15.1	Introduction	233

7.15.2	Minimum Requirements for Inter-band Dual Connectivity	233

7.16	Proximity-based Services	234

7.16.1	Introduction	234

7.16.2	Requirements	234

7.16.2.1	ProSe UE transmission timing	234

7.16.2.1.1	Serving cell or PCell as timing reference	234

7.16.2.1.2	SCell or non-serving cell as timing reference	234

7.16.3	Interruptions with ProSe	234

7.16.3.1	Interruptions at ProSe Direct Discovery configuration	235

7.16.3.2	Interruptions at ProSe Direct Communication configuration	235

7.16.3.3	Interruptions during ProSe Direct Discovery	235

7.16.3.4	Interruptions during ProSe Direct Discovery with discovery gaps	235

7.16.3.5	Interruptions during ProSe Direct Communication	236

7.16.4	Cell reselection for ProSe Direct Discovery on non-serving frequency	236

7.16.4.1	Measurement and evaluation of selected cell	236

7.16.4.2	Measurement of intra-frequency E-UTRAN cells	236

7.16.5	Selection / Reselection of ProSe relay UE	237

7.16.6	ProSe operation under deactivated SCell	237

7.17	Maximum Transmission Timing Difference in Dual Connectivity	238

7.17.1	Introduction	238

7.17.2	Minimum Requirements for maximum transmission timing difference Inter-band Dual Connectivity	238

7.18.1	Introduction	238

7.18.2	SCell Activation Delay Requirement for Deactivated SCell	238

7.18.3	SCell Deactivation Delay Requirement for Activated SCell	238

7.19	Radio Link Monitoring for UE Category M1	238

7.19.1	Introduction	238

7.19.2	Requirements for FD-FDD and TDD CE mode A	239

7.19.2.1	Minimum requirement when no DRX is used	240

7.19.2.2	Minimum requirement when DRX is used	241

7.19.2.3	Minimum requirement at transitions	242

7.19.3	Requirements for HD-FDD with CE mode A	242

7.19.3.1	Minimum requirement when no DRX is used	242

7.19.3.2	Minimum requirement when DRX is used	242

7.19.3.3	Minimum requirement at transitions	244

7.19.4	Requirements for FD-FDD and TDD with CE mode B	244

7.19.4.1	Minimum requirement when no DRX is used	245

7.19.4.2	Minimum requirement when DRX is used	246

7.19.4.3	Minimum requirement at transitions	247

7.19.5	Requirements for HD-FDD with CE mode B	247

7.19.5.1	Minimum requirement when no DRX is used	247

7.19.5.2	Minimum requirement when DRX is used	247

7.19.5.3	Minimum requirement at transitions	249

7.20	UE transmit timing for NB-IoT	249

7.20.1	Introduction	249

7.20.2	Requirements	249

7.21	 UE timer accuracy for NB-IoT	250

7.21.1	Introduction	250

7.21.2	Requirements	250

7.22	Timing Advance for NB-IoT	250

7.22.1	Introduction	250

7.22.2	Requirements	250

7.22.2.1	Timing Advance adjustment delay	250

7.22.2.2	Timing Advance adjustment accuracy	250

7.23	Radio Link Monitoring for Category NB1 UE	250

7.23.1	Introduction	250

7.23.2	Requirements for Category NB1 UE	250

7.23.2.1	Minimum requirement when no DRX is used	251

7.23.2.2	Minimum requirement when DRX is used	251

7.23.2.3	Minimum requirement at transitions	252

7.24	UE transmit timing for Category M1	252

7.24.1	Introduction	252

7.24.2	Requirements	252

7.25	Cell phase synchronization accuracy for MBMS services (FDD)	253

7.25.1	Definition	253

7.25.2	Minimum requirements	253

7.26	UE transmit timing for Category M2	254

7.26.1	Introduction	254

7.26.2	Requirements	254

7.27	UE timer accuracy for category M1	254

7.27.1	Introduction	254

7.27.2	Requirements	254

7.28	Timing Advance for Category M1	254

7.28.1	Introduction	254

7.28.2	Requirements	254

7.29	Interruptions requirements with FeMBMS	254

7.29.1	Introduction	254

7.29.2	Requirements	255

7.30	Numerology switching delay requirements with FeMBMS	255

7.30.1	Introduction	255

7.30.2	Requirements	255

7.31	NR PSCell Addition and Release Delay for E-UTRA - NR Dual Connectivity	255

7.31.1	Introduction	255

7.31.2	NR PSCell Addition Delay Requirement	255

7.31.3	NR PSCell Release Delay Requirement	256

7.32	Interruptions with EN-DC	256

7.32.1	Introduction	256

7.32.2	Requirements	257

7.32.2.1	Interruptions at PSCell addition/release	257

7.32.2.2	Interruptions at transitions between active and non-active during DRX	257

7.32.2.3	Interruptions at transitions from non-DRX to DRX	257

7.32.2.4	Interruptions at SCell addition/release	257

7.32.2.5	Interruptions at SCell activation/deactivation	257

7.32.2.6	Interruptions during measurements on SCC	258

7.32.2.6.1	Interruptions during measurements on deactivated NR SCC	258

7.32.2.6.2	Interruptions during measurements on deactivated E-UTRA SCC	258

7.32.2.7	Interruptions at active BWP switching	258

7.33	Maximum Transmit/Receive Timing Difference in Carrier Aggregation for sTTI and 1ms-TTI with 3 subframe HARQ processing	259

7.33.1	Introduction	259

7.33.2	Requirements	259

7.34	Void	259

7.35	Interruptions with SFTD measurements	259

7.35.1	Introduction	259

7.35.2	Requirements	259

7.36	Interruptions with NE-DC	260

7.32.1	Introduction	260

7.36.2	Requirements	260

7.36.2.1	Interruptions at transitions between active and non-active during DRX	260

7.36.2.2	Interruptions at transitions from non-DRX to DRX	260

7.36.2.3	Interruptions at SCell addition/release	260

7.36.2.4	Interruptions at SCell activation/deactivation	260

7.36.2.5	Interruptions during measurements on SCC	261

7.36.2.5.1	Interruptions during measurements on deactivated NR SCC	261

7.36.2.5.2	Interruptions during measurements on deactivated E-UTRA SCC	261

7.36.2.6	Interruptions at active BWP switching	261

8	UE Measurements Procedures in RRC_CONNECTED State	262

8.1	General Measurement Requirements	262

8.1.1	Introduction	262

8.1.2	Requirements	262

8.1.2.1	UE measurement capability	262

8.1.2.1.1	Monitoring of multiple layers using gaps	268

8.1.2.1.1a	Monitoring of multiple layers using gaps (Increased UE carrier monitoring)	270

8.1.2.1.1b	Monitoring of multiple layers using gaps (E-UTRA-NR dual connectivity)	271

8.1.2.1.1c	Monitoring of multiple layers using gaps (NE-DC)	272

8.1.2.1.2	Network controlled small gap	273

8.1.2.2	E-UTRAN intra frequency measurements	275

8.1.2.2.1	E-UTRAN FDD intra frequency measurements	275

8.1.2.2.2	E-UTRAN TDD intra frequency measurements	279

8.1.2.2.3	E-UTRAN FDD intra frequency measurements with autonomous gaps	283

8.1.2.2.4	E-UTRAN TDD intra frequency measurements with autonomous gaps	284

8.1.2.2.5	E-UTRAN FDD intra-frequency measurements on carrier with FeMBMS/Unicast mixed cells	285

8.1.2.3	E-UTRAN inter frequency measurements	285

8.1.2.3.1	E-UTRAN FDD – FDD inter frequency measurements	286

8.1.2.3.2	E-UTRAN TDD – TDD inter frequency measurements	291

8.1.2.3.3	E-UTRAN TDD – FDD inter frequency measurements	298

8.1.2.3.4	E-UTRAN FDD – TDD inter frequency measurements	298

8.1.2.3.5	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	298

8.1.2.3.6	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	299

8.1.2.3.7	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	301

8.1.2.3.8	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	302

8.1.2.3.9	E-UTRAN FDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	303

8.1.2.3.10	E-UTRAN TDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	310

8.1.2.4	Inter RAT measurements	310

8.1.2.4.1	E-UTRAN FDD – UTRAN FDD measurements	310

8.1.2.4.2	E-UTRAN TDD – UTRAN FDD measurements	315

8.1.2.4.3	E-UTRAN TDD – UTRAN TDD measurements	315

8.1.2.4.4	E-UTRAN FDD – UTRAN TDD measurements	319

8.1.2.4.5	E-UTRAN FDD – GSM measurements	319

8.1.2.4.6	E-UTRAN TDD – GSM measurements	324

8.1.2.4.7	E-UTRAN FDD – UTRAN FDD measurements for SON	324

8.1.2.4.8	E-UTRAN TDD – UTRAN FDD measurements for SON	326

8.1.2.4.9	E-UTRAN FDD – cdma2000 1xRTT measurements	326

8.1.2.4.9.1A	E-UTRAN FDD – cdma2000 1xRTT measurements when no DRX is used	326

8.1.2.4.10	E-UTRAN TDD – cdma2000 1xRTT measurements	327

8.1.2.4.11	E-UTRAN FDD – HRPD measurements	327

8.1.2.4.12	E-UTRAN TDD – HRPD measurements	327

8.1.2.4.13	E-UTRAN TDD – UTRAN TDD measurements for SON	327

8.1.2.4.14	E-UTRAN FDD – UTRAN TDD measurements for SON	329

8.1.2.4.15	E-UTRAN FDD – cdma2000 1xRTT measurements for SON ANR	329

8.1.2.4.16	E-UTRAN TDD – cdma2000 1xRTT measurements for SON ANR	329

8.1.2.4.17	E-UTRAN FDD-UTRAN FDD measurements with autonomous gaps	329

8.1.2.4.18	E-UTRAN TDD-UTRAN FDD measurements with autonomous gaps	330

8.1.2.4.19	E-UTRAN FDD – WLAN measurements	330

8.1.2.4.20	E-UTRAN TDD – WLAN measurements	332

8.1.2.4.21	E-UTRAN FDD – NR measurements	332

8.1.2.4.22	E-UTRAN TDD – NR measurements	335

8.1.2.4.23	Void	335

8.1.2.4.24	Void	335

8.1.2.4.25	E-UTRAN FDD – NR SFTD Measurements	335

8.1.2.4.26	E-UTRAN TDD – NR SFTD Measurements	337

8.1.2.5	E-UTRAN OTDOA Intra-Frequency RSTD Measurements	337

8.1.2.5.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	337

8.1.2.5.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	339

8.1.2.5.3	E-UTRAN FDD Intra-Frequency OTDOA Measurements for UE Category 1bis	341

8.1.2.5.4	E-UTRAN TDD Intra-Frequency OTDOA Measurements for UE Category 1bis	342

8.1.2.6.5	Void	344

8.1.2.6.6	Void	344

8.1.2.6.7	Void	344

8.1.2.6.8	Void	344

8.1.2.6	E-UTRAN Inter-Frequency OTDOA Measurements	344

8.1.2.6.1	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements	345

8.1.2.6.2	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements	346

8.1.2.6.3	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements	348

8.1.2.6.4	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements	350

8.1.2.6.5	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	351

8.1.2.6.6	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	353

8.1.2.6.7	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	355

8.1.2.6.8	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	357

8.1.2.7	E-UTRAN E-CID Measurements	358

8.1.2.7.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	358

8.1.2.7.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	360

8.1.2.7.3	E-UTRAN FDD Intra-frequency E-CID RSRP and RSRQ Measurements	361

8.1.2.7.4	E-UTRAN TDD Intra-frequency E-CID RSRP and RSRQ Measurements	362

8.1.2.8	E-UTRAN intra-frequency measurements under time domain measurement resource restriction	362

8.1.2.8.1	E-UTRAN FDD intra-frequency measurements	362

8.1.2.8.2	E-UTRAN TDD intra-frequency measurements	365

8.1.2.8.3	E-UTRAN FDD intra-frequency measurements with CRS assistance information	368

8.1.2.8.4	E-UTRAN TDD intra-frequency measurements with CRS assistance infromation	371

8.1.2.9	E-UTRAN E-CID Measurements when Time Domain Measurement Resource Restriction Pattern is Configured	375

8.1.2.9.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	375

8.1.2.9.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	375

8.1.2.9.3	E-UTRAN FDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	376

8.1.2.9.4	E-UTRAN TDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	376

8.1.2.10	Void	377

8.2	Capabilities for Support of Event Triggering and Reporting Criteria	377

8.2.1	Introduction	377

8.2.2	Requirements	377

8.3	Measurements for E-UTRA carrier aggregation	381

8.3.1	Introduction	381

8.3.2	Measurements of the primary component carrier	381

8.3.3	Measurements of a secondary component carrier	381

8.3.3.1	Measurements of a secondary component carrier with active SCell	381

8.3.3.2	Measurements of a secondary component carrier with deactivated SCell	382

8.3.3.2.1	E-UTRAN secondary component carrier measurements when no common DRX is used	382

8.3.3.2.2	E-UTRAN secondary component carrier measurements when common DRX is used	383

8.3.3.3	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and activated SCell	384

8.3.3.4	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and deactivated SCell	385

8.4	OTDOA RSTD Measurements for E-UTRAN carrier aggregation	385

8.4.1	Introduction	385

8.4.2	Measurements on the primary component carrier	385

8.4.3	Measurements on a secondary component carrier	386

8.4.4	Measurements on both primary component carrier and a secondary component carrier	387

8.4.5	Measurements on different secondary component carriers	388

8.5	Measurements for UE category 0	389

8.5.1	Introduction	389

8.5.2 	Requirements	389

8.5.2.1 	E-UTRAN intra frequency measurements	389

8.5.2.1.1 	E-UTRAN FDD intra frequency measurements	389

8.5.2.1.2 	E-UTRAN intra frequency measurements for HD-FDD	392

8.5.2.1.3	E-UTRAN TDD intra frequency measurements	394

8.5.2.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category 0	398

8.5.2.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category 0	398

8.5.2.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category 0	399

8.6	Discovery signal measurements	400

8.6.1	Introduction	400

8.6.2	Requirements for CRS based discovery signal measurements	400

8.6.2.1	E-UTRAN intra frequency measurements	400

8.6.2.1.1	E-UTRAN FDD intra frequency measurements	400

8.6.2.1.2	E-UTRAN TDD intra frequency measurements	403

8.6.2.2	E-UTRAN inter frequency measurements	405

8.6.2.2.1	E-UTRAN FDD – FDD inter-frequency measurements	406

8.6.2.2.2	E-UTRAN TDD – TDD inter frequency measurements	408

8.6.2.2.3	E-UTRAN TDD – FDD inter frequency measurements	411

8.6.2.2.4	E-UTRAN FDD – TDD inter frequency measurements	411

8.6.3	Requirements for CSI-RS based discovery signal measurements	411

8.6.3.1	E-UTRAN intra frequency measurements	411

8.6.3.1.1	E-UTRAN FDD intra frequency measurements	411

8.6.3.1.2	E-UTRAN TDD intra frequency measurements	414

8.6.3.2	E-UTRAN inter frequency measurements	416

8.6.3.2.1	E-UTRAN FDD – FDD inter frequency measurements	416

8.6.3.2.2	E-UTRAN TDD – TDD inter frequency measurements	419

8.6.3.2.3	E-UTRAN TDD – FDD inter frequency measurements	422

8.6.3.2.4	E-UTRAN FDD – TDD inter frequency measurements	422

8.7	Discovery signal measurements for E-UTRA carrier aggregation	422

8.7.1	Introduction	422

8.7.2	Requirements for CRS based discovery signal measurements for E-UTRA carrier aggregation	422

8.7.2.1	Measurements of the primary component carrier	422

8.7.2.2	Measurements of a secondary component carrier	422

8.7.2.3	Measurements of a secondary component carrier with active SCell	422

8.7.2.4	Measurements of a secondary component carrier with deactivated SCell	423

8.7.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	423

8.7.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	424

8.7.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	426

8.7.3.1	Measurements of the primary component carrier	426

8.7.3.2	Measurements of a secondary component carrier	426

8.7.3.3	Measurements of a secondary component carrier with active SCell	426

8.7.3.4	Measurements of a secondary component carrier with deactivated SCell	426

8.7.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	426

8.7.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	427

8.8	Measurements for E-UTRA dual connectivity	429

8.8.1	Introduction	429

8.8.2	Intra-frequency measurements requirements on PCell	429

8.8.3	Intra-frequency measurements requirements on PSCell	429

8.8.4	Inter-frequency and inter-RAT measurement requirements	429

8.8.5	Intra-frequency measurements with autonomous gaps	430

8.8.5.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	430

8.8.5.2	ECGI reporting delay	431

8.8.6	Inter-frequency measurements with autonomous gaps	431

8.8.6.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	431

8.8.6.2	ECGI reporting delay	432

8.8.7	SSTD Measurements	432

8.8.7.1	Introduction	432

8.8.7.2	SSTD Measurement requirements	432

8.8.7.3	SSTD Measurement Reporting Delay	432

8.8.8	Intra-frequency measurements requirements on SCell	433

8.9	MBSFN Measurements	433

8.9.1	Introduction	433

8.9.2	MBSFN RSRP Measurements	433

8.9.3	MBSFN RSRQ Measurements	433

8.9.4	MCH BLER Measurements	434

8.10	Proximity-based Services	434

8.10.1	Introduction	434

8.10.2	Requirements	434

8.10.2.1	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	434

8.10.2.2	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	434

8.11	Discovery Signal Measurements under Operation with Frame Structure 3	435

8.11.1	Introduction	435

8.11.2	CRS based discovery signal measurements	435

8.11.2.1	E-UTRAN intra-frequency measurements	435

8.11.2.1.1	Requirements	436

8.11.2.1.1.1	Requirements when no DRX is used	436

8.11.2.1.1.1.1	Measurement Reporting Requirements	437

8.11.2.1.1.2	Requirements when DRX is used	438

8.11.2.1.1.2.1	Measurement Reporting Requirements	440

8.11.2.2	E-UTRAN inter-frequency measurements	441

8.11.2.2.1	E-UTRAN FDD-FS3 inter-frequency measurements	441

8.11.2.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	445

8.11.3	CSI-RS based discovery signal measurements	445

8.11.3.1	E-UTRAN intra-frequency measurements	445

8.11.3.1.1	Requirements	445

8.11.3.1.1.1	Requirements when no DRX is used	445

8.11.3.1.1.1.1	Measurement Reporting Requirements	447

8.11.3.1.1.2	Requirements when DRX is used	447

8.11.3.1.1.2.1	Measurement Reporting Requirements	448

8.11.3.2	E-UTRAN inter-frequency measurements	449

8.11.3.2.1	E-UTRAN FDD – FS3 inter-frequency measurements	449

8.11.3.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	453

8.11.4	RSSI measurements	453

8.11.4.1	E-UTRAN intra-frequency measurements	453

8.11.4.2	E-UTRAN inter-frequency measurements	453

8.11.5	Channel occupancy measurements	453

8.11.5.1	E-UTRAN intra-frequency channel occupancy measurements	453

8.11.5.2	E-UTRAN inter-frequency channel occupancy measurements	454

8.12	Discovery Signal Measurements for E-UTRA Carrier Aggregation under Operation with Frame Structure 3	454

8.12.1	Introduction	454

8.12.2	CRS based discovery signal measurements for E-UTRA carrier aggregation	454

8.12.2.1	Introduction	454

8.12.2.2	Measurements of a secondary component carrier	454

8.12.2.3	Measurements of a secondary component carrier with active SCell	454

8.12.2.4	Measurements of a secondary component carrier with deactivated SCell	454

8.12.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	455

8.12.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	457

8.12.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	460

8.12.3.1	Introduction	460

8.12.3.2	Measurements of a secondary component carrier	460

8.12.3.3	Measurements of a secondary component carrier with active SCell	460

8.12.3.4	Measurements of a secondary component carrier with deactivated SCell	460

8.12.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	460

8.12.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	462

8.13	Measurements for UE Category M1	464

8.13.1	Introduction	464

8.13.2	Requirements for UE category M1 with CE mode A	464

8.13.2.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode A	465

8.13.2.1.1	E-UTRAN FDD intra frequency measurements	465

8.13.2.1.2	E-UTRAN intra frequency measurements for HD-FDD	469

8.13.2.1.3	E-UTRAN TDD intra frequency measurements	471

8.13.2.2	Void	475

8.13.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	475

8.13.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	475

8.13.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	478

8.13.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	480

8.13.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	481

8.13.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	481

8.13.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	483

8.13.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	485

8.13.2.5	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode A	486

8.13.2.5.1	Intra-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	486

8.13.2.5.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	486

8.13.2.5.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	487

8.13.2.5.4	Inter-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	487

8.13.2.5.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	487

8.13.2.5.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	488

8.13.2.5.7	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	488

8.13.2.5.8	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	489

8.13.2.5.9	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	490

8.13.2.6	E-UTRAN inter frequency measurements by UE category M1 with CE mode A	490

8.13.2.6.1	E-UTRAN FDD - FDD inter frequency measurements	491

8.13.2.6.2	E-UTRAN inter-frequency measurements for HD-FDD	495

8.13.2.6.3	E-UTRAN TDD inter frequency measurements	497

8.13.2.7	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode A	501

8.13.3	Requirements for UE category M1 with CE mode B	501

8.13.3.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode B	501

8.13.3.1.1	E-UTRAN FDD intra frequency measurements	501

8.13.3.1.2	E-UTRAN intra frequency measurements for HD-FDD	506

8.13.3.1.3	E-UTRAN TDD intra frequency measurements	508

8.13.3.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	514

8.13.3.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category M1 with CE mode B	515

8.13.3.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	515

8.13.3.2	Void	516

8.13.3.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	516

8.13.3.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	516

8.13.3.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	519

8.13.3.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	521

8.13.3.4	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode B	522

8.13.3.4.1	Intra-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	522

8.13.3.4.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	522

8.13.3.4.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	522

8.13.3.4.4	Inter-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	523

8.13.3.4.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	523

8.13.3.4.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	524

8.13.3.5	E-UTRAN inter frequency measurements by UE category M1 with CE Mode B	524

8.13.3.5.1	E-UTRAN FDD - FDD inter frequency measurements	524

8.13.3.5.2 	E-UTRAN inter-frequency measurements for HD-FDD	528

8.13.3.5.3	E-UTRAN TDD inter frequency measurements	531

8.13.3.6	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode B	535

8.13.3.7	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	536

8.13.3.7.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	536

8.13.3.7.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	538

8.13.3.7.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	540

8.14	Measurements for UE category NB1	541

8.14.1	Introduction	541

8.14.2	NB-IoT intra frequency measurements under normal coverage	541

8.14.2.1	NB-IoT intra frequency measurements when no DRX is used	541

8.14.2.2	NB-IoT intra frequency measurements when DRX is used	541

8.14.3	NB-IoT intra frequency measurements under enhanced coverage	541

8.14.3.1	NB-IoT intra frequency measurements when no DRX is used	541

8.14.3.2	NB-IoT intra frequency measurements when DRX is used	541

8.15	Void	542

8.16	Measurements for UE Category M2	542

8.16.1	Introduction	542

8.16.2	Requirements for UE category M2 with CE mode A	542

8.16.2.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	542

8.16.2.1.1	UE Rx-Tx Measurement Reporting Delay	543

8.16.2.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	543

8.16.2.2.1	UE Rx-Tx Measurement Reporting Delay	544

8.16.2.2a	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	544

8.16.2.2a.1	UE Rx-Tx Measurement Reporting Delay	544

8.16.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	545

8.16.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	545

8.16.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	547

8.16.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	550

8.16.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	550

8.16.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	550

8.16.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	552

8.16.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	555

8.16.3	Requirements for UE category M2 with CE mode B	555

8.16.3.1	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	555

8.16.3.1.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	555

8.16.3.1.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	558

8.16.3.1.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	560

8.16.3.2	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	561

8.16.3.2.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	561

8.16.3.2.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	563

8.16.3.2.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	565

8.17	Measurements for E-UTRA – NR Dual Connectivity	566

8.17.1	Introduction	566

8.17.1.1	Measurement Gap Sharing	566

8.17.1A	Intrafrequency Measurements	567

8.17.2	SFTD Measurements	567

8.17.2.1	Introduction	567

8.17.2.2	SFTD Measurement requirements	567

8.17.2.3	SFTD Measurement Reporting Delay	567

8.17.3	E-UTRA Inter-frequency Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	568

8.17.3.1	Introduction	568

8.17.3.2	E-UTRAN FDD inter frequency measurements	568

8.17.3.2.1	E-UTRAN FDD inter frequency measurements when no DRX is used	568

8.17.3.2.2	E-UTRAN FDD inter frequency measurements when DRX is used	569

8.17.3.3	E-UTRAN TDD inter frequency measurements	571

8.17.3.3.1	E-UTRAN TDD inter frequency measurements when no DRX is used	571

8.17.3.3.2	E-UTRAN TDD inter frequency measurements when DRX is used	573

8.17.4	E-UTRA Inter-RAT NR Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	575

8.17.4.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	575

8.17.4.1.1	NR Inter-RAT cell identification	575

8.17.4.1.2	NR Inter-RAT measurement	577

8.17.4.1.3	NR Inter-RAT measurement reporting	578

8.17.4.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	578

8.17.5	E-UTRAN FDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	578

8.17.5.1	Introduction	578

8.17.5.2	E-UTRAN FDD – UTRAN FDD measurements when no DRX is used	579

8.17.5.2.1	Identification of a new UTRA FDD cell	579

8.17.5.2.2	Enhanced UTRA FDD cell identification requirements	579

8.17.5.2.3	UE UTRA FDD CPICH measurement capability	579

8.17.5.2.4	Periodic Reporting	580

8.17.5.2.5	Event Triggered Reporting	580

8.17.5.2.6	Event-triggered Periodic Reporting	580

8.17.5.3	E-UTRAN FDD – UTRAN FDD measurements when DRX is used	580

8.17.5.3.1	Periodic Reporting	581

8.17.5.3.2	Event Triggered Reporting	581

8.17.5.3.3	Event-triggered Periodic Reporting	582

8.17.6	E-UTRAN TDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	582

8.17.7	E-UTRAN FDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	582

8.17.7.1	Introduction	582

8.17.7.2	Identification of a new UTRA FDD cell for SON	582

8.17.7.2.1	Requirements when no DRX is used	582

8.17.7.2.2	Requirements when DRX is used	583

8.17.7.2.3	Reporting Delay	583

8.17.8	E-UTRAN TDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	584

8.17.9	E-UTRAN TDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	584

8.17.9.1	Introduction	584

8.17.9.2	E-UTRAN FDD – UTRAN TDD measurements when no DRX is used	584

8.17.9.2.1	Identification of a new UTRA FDD cell	584

8.17.9.2.2	Enhanced UTRA TDD cell identification requirements	584

8.17.9.2.3	UE UTRA TDD P-CCPCH RSCP measurement capability	585

8.17.9.2.4	Periodic Reporting	585

8.17.9.2.5	Event Triggered Reporting	585

8.17.9.2.6	Event-triggered Periodic Reporting	586

8.17.9.3	E-UTRAN TDD – UTRAN TDD measurements when DRX is used	586

8.17.9.3.1	Periodic Reporting	587

8.17.9.3.2	Event Triggered Reporting	587

8.17.9.3.3	Event-triggered Periodic Reporting	587

8.17.10	E-UTRAN FDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	588

8.17.11	E-UTRAN TDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	588

8.17.11.1	Introduction	588

8.17.11.2	Identification of a new UTRA TDD cell for SON	588

8.17.11.2.1	Requirements when no DRX is used	588

8.17.11.2.2	Requirements when DRX is used	588

8.17.11.2.3	Reporting Delay	589

8.17.12	E-UTRAN FDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	589

8.17.13	E-UTRAN FDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	590

8.17.13.1	Introduction	590

8.17.13.2	E-UTRAN FDD – GSM measurements when no DRX is used	590

8.17.13.2.1	GSM carrier RSSI	590

8.17.13.2.2	BSIC verification	590

8.17.13.2.3	Enhanced BSIC verification	592

8.17.13.2.4	Periodic Reporting	592

8.17.13.2.5	Event Triggered Reporting	592

8.17.13.2.6	Event-triggered Periodic Reporting	593

8.17.13.3	E-UTRAN FDD – GSM measurements when DRX is used	593

8.17.13.3.1	GSM carrier RSSI	593

8.17.13.3.2	BSIC verification	593

8.17.13.3.3	Periodic Reporting	595

8.17.13.3.4	Event Triggered Reporting	595

8.17.13.3.5	Event-triggered Periodic Reporting	595

8.17.14	E-UTRAN TDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	595

8.18	Measurements for non-BL/CE UE	595

8.18.1	Introduction	595

8.18.2	Requirements for non-BL/CE UE with CE Mode B	595

8.18.2.1	E-UTRAN intra frequency measurements	595

8.18.2.1.1	E-UTRAN FDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	595

8.18.2.1.2	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD non-BL/CE with CE Mode B	596

8.18.2.1.3	E-UTRAN TDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	596

8.19	Measurements for NR – E-UTRA Dual Connectivity	597

8.19.1	Introduction	597

8.19.2	Intra-frequency Measurements	598

8.19.3	Inter-frequency Measurements	598

8.19.4	Intra-frequency RSTD Measurements	598

8.19.5	Intra-frequency E-CID Measurements	598

9	Measurements performance requirements for UE	598

9.1	E-UTRAN measurements	599

9.1.1	Introduction	599

9.1.2	Intra-frequency RSRP Accuracy Requirements	599

9.1.2.1	Absolute RSRP Accuracy	599

9.1.2.2	Relative Accuracy of RSRP	600

9.1.2.3	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction	601

9.1.2.4	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction	601

9.1.2.5	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	602

9.1.2.6	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction with CRS assistance information	603

9.1.2.7	Absolute RSRP Accuracy for UE Category 1bis	604

9.1.2.8	Relative Accuracy of RSRP for UE Category 1bis	605

9.1.2A	Intra-frequency RSRP Accuracy Requirements in High Doppler Conditions	606

9.1.2A.1	Absolute RSRP Accuracy in high Doppler conditions	606

9.1.2A.2	Relative Accuracy of RSRP in high Doppler conditions	606

9.1.2B	Intra-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	607

9.1.2B.1	Introduction	607

9.1.2B.2	Intra-frequency Absolute RSRP Accuracy for CA Idle Mode Measurements	607

9.1.3	Inter-frequency RSRP Accuracy Requirements	608

9.1.3.1	Absolute RSRP Accuracy	608

9.1.3.2	Relative Accuracy of RSRP	609

9.1.3.3	Absolute RSRP Accuracy for UE Category 1bis	610

9.1.3.4	Relative Accuracy of RSRP for UE Category 1bis	610

9.1.3A	Inter-frequency RSRP Accuracy Requirements in High Doppler Conditions	611

9.1.3A.1	Absolute RSRP Accuracy in high Doppler conditions	611

9.1.3A.2	Relative Accuracy of RSRP in high Doppler conditions	612

9.1.3B	Inter-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	613

9.1.3B.1	Introduction	613

9.1.3B.2	Inter-frequency Absolute RSRP Accuracy for Overlapping Carrier	613

9.1.3B.3	Inter-frequency Absolute RSRP Accuracy for Overlapping and Non-overlapping Carrier	613

9.1.4	RSRP Measurement Report Mapping	614

9.1.5	Intra-frequency RSRQ Accuracy Requirements	614

9.1.5.1	Absolute RSRQ Accuracy	614

9.1.5.2	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction	615

9.1.5.3	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	616

9.1.5.4	Absolute WB-RSRQ Accuracy	617

9.1.5.5	Absolute RSRQ Accuracy for UE Category 1bis	618

9.1.5A	Intra-frequency RSRQ Accuracy Requirements in High Doppler Conditions	619

9.1.5A.1	Absolute RSRQ Accuracy in high Doppler conditions	619

9.1.5B	Intra-frequency RSRQ Accuracy requirements for CA Idle Mode Measurements	619

9.1.5B.1	Introduction	619

9.1.5B.2	Intra-frequency Absolute RSRQ Accuracy for CA Idle Mode Measurements	619

9.1.6	Inter-frequency RSRQ Accuracy Requirements	620

9.1.6.1	Absolute RSRQ Accuracy	620

9.1.6.2	Relative Accuracy of RSRQ	621

9.1.6.3	Absolute WB-RSRQ Accuracy	622

9.1.6.4	Relative WB-RSRQ Accuracy	622

9.1.6.5	Absolute RSRQ Accuracy for UE Category 1bis	623

9.1.6.6	Relative Accuracy of RSRQ for UE Category 1bis	624

9.1.6A	Inter-frequency RSRQ Accuracy Requirements in High Doppler Conditions	624

9.1.6A.1	Absolute RSRQ Accuracy in high Doppler conditions	624

9.1.6A.2	Relative Accuracy of RSRQ in high Doppler conditions	625

9.1.6B	Inter-frequency absolute RSRQ Accuracy requirements for CA Idle Mode Measurements	626

9.1.6B.1	Introduction	626

9.1.6B.2	Inter-frequency Absolute RSRQ Accuracy for Overlapping Carrier	626

9.1.6B.3	Inter-frequency absolute RSRQ Accuracy for Overlapping and Non-overlapping Carrier	626

9.1.7	RSRQ Measurement Report Mapping	627

9.1.8	Power Headroom	627

9.1.8.1	Period	628

9.1.8.2	Reporting Delay	628

9.1.8.3	Void	628

9.1.8.4	Report Mapping	628

9.1.9	UE Rx – Tx time difference	628

9.1.9.1	Measurement Requirement	628

9.1.9.2	Measurement Report mapping	629

9.1.9.3	Measurement Requirement under Time Domain Measurement Resource Restriction	630

9.1.9.4	Measurement Requirement when Time Domain Measurement Resource Restriction Pattern is Configured with CRS Assistance Information	631

9.1.10	Reference Signal Time Difference (RSTD)	632

9.1.10.1	Intra-Frequency Accuracy Requirement	632

9.1.10.2	Inter-Frequency Accuracy Requirement	633

9.1.10.3	RSTD Measurement Report Mapping	635

9.1.10.4	Higher-Resolution RSTD Measurement Report Mapping	635

9.1.10.5	Intra-Frequency Accuracy Requirement for UE Category 1bis	636

9.1.10.6	Inter-Frequency Accuracy Requirement for UE Category 1bis	637

9.1.11	Carrier aggregation measurement accuracy	638

9.1.11.1	Primary component carrier accuracy requirement	639

9.1.11.2	Secondary component carrier accuracy requirement	639

9.1.11.3	Primary and secondary component carrier relative accuracy requirement	639

9.1.11.4	Secondary component carrier relative accuracy requirement	639

9.1.12	Reference Signal Time Difference (RSTD) Measurement Accuracy Requirements for Carrier Aggregation	639

9.1.13	Measurement accuracy for UE category 0	640

9.1.13.1	Intra-frequency Absolute RSRP Accuracy for UE category 0	640

9.1.13.2	Intra-frequency Relative Accuracy of RSRP for UE category 0	640

9.1.13.3	Intra-frequency Absolute RSRQ Accuracy for UE category 0	641

9.1.14	Accuracy requirements for Discovery Signal Measurements	642

9.1.14.1	Introduction	642

9.1.14.2	RSRP measurements in discovery signal occasions	642

9.1.14.3	CSI-RSRP measurements in discovery signal occasions	642

9.1.14.3.1	Intra-frequency CSI-RSRP measurements	642

9.1.14.3.1.1	Absolute CSI-RSRP measurement requirements	642

9.1.14.3.1.2	Relative CSI-RSRP measurement requirements	643

9.1.14.3.2	Inter-frequency CSI-RSRP measurements	644

9.1.14.3.2.1	Absolute CSI-RSRP measurement requirements	644

9.1.14.3.2.2	Relative CSI-RSRP measurement requirements	644

9.1.14.3.3	CSI-RSRP measurement report mapping	645

9.1.14.4	RSRQ measurements in discovery signal occasions	645

9.1.15	Discovery signal measurements accuracy for E-UTRAN carrier aggregation	645

9.1.15.1	Requirements for CRS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	646

9.1.15.1.1	Primary component carrier accuracy requirement	646

9.1.15.1.2	Secondary component carrier accuracy requirement	646

9.1.15.1.3	Primary and secondary component carrier relative accuracy requirement	646

9.1.15.1.4	Secondary component carrier relative accuracy requirement	646

9.1.15.2	Requirements for CSI-RS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	646

9.1.15.2.1	Primary component carrier accuracy requirement	646

9.1.15.2.2	Secondary component carrier accuracy requirement	646

9.1.15.2.3	Primary and secondary component carrier relative accuracy requirement	646

9.1.15.2.4	Secondary component carrier relative accuracy requirement	646

9.1.16	Accuracy requirements for RSRQ measurement on all OFDM symbols	647

9.1.17	RS-SINR Measurements	647

9.1.17.1	Measurement Report Mapping	647

9.1.17.2	Intra-frequency RS-SINR Measurement Accuracy Requirements	648

9.1.17.2.1	Absolute RS-SINR Measurement Accuracy Requirements	648

9.1.17.3	Inter-frequency RS-SINR Measurement Accuracy Requirements	648

9.1.17.3.1	Absolute RS-SINR Measurement Accuracy Requirements	648

9.1.17.3.2	Relative RS-SINR Measurement Accuracy Requirements	649

9.1.18	Accuracy Requirements for Measurements under Operation with Frame Structure 3	650

9.1.18.1	Introduction	650

9.1.18.2	RSRP measurements	650

9.1.18.2.1	RSRP measurement report mapping	650

9.1.18.2.2	Inter-frequency absolute RSRP measurement accuracy requirements	650

9.1.18.2.3	Inter-frequency relative RSRP measurement accuracy requirements	651

9.1.18.2.4	Intra-frequency absolute RSRP measurement accuracy requirements	651

9.1.18.2.5	Intra-frequency relative RSRP measurement accuracy requirements	652

9.1.18.3	RSRQ measurements	652

9.1.18.3.1	RSRQ measurement report mapping	652

9.1.18.3.2	Inter-frequency absolute RSRQ measurement accuracy requirements	652

9.1.18.3.3	Inter-frequency relative RSRQ measurement accuracy requirements	653

9.1.18.3.4	Intra-frequency absolute RSRQ measurement accuracy requirements	653

9.1.18.4	CSI-RSRP measurements	654

9.1.18.4.1	CSI-RSRP measurement report mapping	654

9.1.18.4.2	Inter-frequency absolute CSI-RSRP measurement accuracy requirements	654

9.1.18.4.3	Inter-frequency relative CSI-RSRP measurement accuracy requirements	654

9.1.18.4.4	Intra-frequency absolute CSI-RSRP measurement accuracy requirements	655

9.1.18.4.5	Intra-frequency relative CSI-RSRP measurement accuracy requirements	655

9.1.18.5	RSSI measurements	656

9.1.18.5.1	RSSI measurement report mapping	656

9.1.18.5.2	Intra-frequency absolute RSSI measurement accuracy requirements	656

9.1.18.5.3	Inter-frequency absolute RSSI measurement accuracy requirements	657

9.1.18.6	Channel occupancy measurements	657

9.1.18.6.1	Intra-frequency channel occupancy measurement accuracy requirements	657

9.1.18.6.2	Inter-frequency channel occupancy measurement accuracy requirements	657

9.1.19	Accuracy Requirements for Carrier Aggregation for Measurements under Operation with Frame Structure 3	658

9.1.19.1	Introduction	658

9.1.19.2	Accuracy requirements for measurements on SCC	658

9.1.19.3	Relative accuracy requirements for measurements on different SCCs	658

9.1.19.4	Relative accuracy requirements for measurements on SCC and PCC	658

9.1.20	SFN and Subframe Time Difference (SSTD)	659

9.1.20.1	SSTD Accuracy Requirement	659

9.1.20.2	SSTD Measurement Report Mapping	659

9.1.21	Measurement accuracy for UE category M1	660

9.1.21.1	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	660

9.1.21.2	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	661

9.1.21.3	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	662

9.1.21.4	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	663

9.1.21.5	RSRP Measurement Report Mapping	664

9.1.21.6	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode A	664

9.1.21.7	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode B	665

9.1.21.8	RSRQ Measurement Report Mapping	666

9.1.21.9	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	666

9.1.21.10	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	667

9.1.21.11	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	668

9.1.21.12	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	669

9.1.21.13 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode A	671

9.1.21.14	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode A	671

9.1.21.15 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode B	672

9.1.21.16	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode B	673

9.1.21.17	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	675

9.1.21.18	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	675

9.1.21.19	UE RX-TX time difference Accuracy Requirement for Cat-M1	676

9.1.21.20	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	677

9.1.21.21	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	680

9.1.22	Measurement accuracy for UE Category NB1	683

9.1.22.1	Intra-frequency Absolute NRSRP Accuracy for UE Category NB1	683

9.1.22.2	Void	684

9.1.22.3	Intra-frequency Absolute NRSRQ Accuracy for UE Category NB1	684

9.1.22.4	Void	685

9.1.22.5	Inter-frequency Absolute NRSRP Accuracy for UE Category NB1	685

9.1.22.6	Void	686

9.1.22.7	Inter-frequency Absolute NRSRQ Accuracy for UE Category NB1	686

9.1.22.8	Void	687

9.1.22.9	NRSRP Measurement Report Mapping	687

9.1.22.10	Intra-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	687

9.1.22.11	Inter-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	688

9.1.22.12	Intra-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	689

9.1.22.13	Inter-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	690

9.1.22.14	NRSRQ Measurement Report Mapping	691

9.1.22.15	MSG3-based Measurement Report Mapping for UE Category NB1	692

9.1.22.16	Downlink Channel Quality Measurement Accuracy for UE Category NB1	692

9.1.23	Power Headroom for UE Category NB1	693

9.1.23.1	Period	693

9.1.23.2	Reporting Delay	693

9.1.23.3	Report Mapping for UE Category NB1	693

9.1.23.3.1	Void	695

9.1.23.3.2	Void	695

9.1.23.4	Report Mapping for UE Category NB1 for UE Power Class 6	695

9.1.24	Void	696

9.1.25	Measurement accuracy for UE category M2	696

9.1.25.1	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	696

9.1.25.2	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	697

9.1.25.3	UE RX-TX time difference Accuracy Requirement for Cat-M2	698

9.1.25.4	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	699

9.1.25.5	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	701

9.1.26	Measurement Accuracy for non-BL CE UE	702

9.1.26.1	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	703

9.1.26.2	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	703

9.1.26.3	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	704

9.1.26.4	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	705

9.1.26.5	RSRP Measurement Report Mapping	706

9.1.26.6	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	706

9.1.26.7	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	706

9.1.26.8	RSRQ Measurement Report Mapping	707

9.1.26.9	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	707

9.1.26.10	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	708

9.1.26.11	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	709

9.1.26.12	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	710

9.1.26.13	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	711

9.1.26.14	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode A	711

9.1.26.15	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	711

9.1.26.16	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode B	712

9.1.27	SFN and frame Timing Difference (SFTD)	713

9.1.27.1	SFTD Accuracy Requirement	713

9.2	UTRAN FDD Measurements	715

9.2.1	UTRAN FDD CPICH RSCP	715

9.2.2	Void	716

9.2.3	UTRAN FDD CPICH Ec/No	716

9.3	UTRAN TDD Measurements	716

9.3.1	UTRAN TDD P-CCPCH RSCP	717

9.3.2	Void	717

9.3.3	Void	717

9.4	GSM Measurements	717

9.4.1	GSM carrier RSSI	717

9.5	CDMA2000 1x RTT Measurements	717

9.5.1	CDMA2000 1x RTT Pilot Strength	717

9.6	PCMAX,c	718

9.6.1	Report Mapping	718

9.6.2 	Estimation Period	718

9.6.3	Reporting Delay	718

9.7	IEEE802.11 Measurements	718

9.7.1	WLAN RSSI	718

9.7.2	WLAN RSSI Measurement Report Mapping	718

9.8	MBSFN Measurements	719

9.8.1	Introduction	719

9.8.2	MBSFN RSRP	719

9.8.2.1	Absolute MBSFN RSRP measurement accuracy requirements	719

9.8.2.2	MBSFN RSRP measurement report mapping	720

9.8.2.3	MBSFN RSRP measurement report mapping for 7.5 kHz subcarrier spacing	720

9.8.2.4	MBSFN RSRP measurement report mapping for 1.25 kHz subcarrier spacing	721

9.8.3	MBSFN RSRQ	721

9.8.3.1	Absolute MBSFN RSRQ measurement accuracy requirements	721

9.8.3.2	MBSFN RSRQ measurement report mapping	722

9.8.3.3	MBSFN RSRQ measurement report mapping for 7.5 kHz subcarrier spacing	722

9.8.3.4	MBSFN RSRQ measurement report mapping for 1.25 kHz subcarrier spacing	723

9.8.4	MCH BLER	723

9.8.4.1	Measurement report mapping for MCH BLER	723

9.8.4.2	Measurement report mapping for MCH Block Number	724

9.9	ProSe Measurements	725

9.9.1	Introduction	725

9.9.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	725

9.9.2.1	Absolute S-RSRP Accuracy	725

9.9.2.2	Relative Accuracy of S-RSRP	726

9.9.3	Intra-Frequency SD-RSRP Measurement Accuracy Requirements	727

9.9.3.1	Absolute SD-RSRP Accuracy	727

9.9.3.2	Relative Accuracy of SD-RSRP	727

9.10	V2X Measurements	728

9.10.1	Introduction	728

9.10.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	728

9.10.2.1	Absolute S-RSRP Accuracy	728

9.10.2.2	Relative Accuracy of S-RSRP	729

9.10.3	PSSCH-RSRP Measurement Accuracy Requirements	729

9.10.3.1	Intra-frequency Absolute PSSCH-RSRP Accuracy	729

9.10.4	S-RSSI Measurement Accuracy Requirements	730

9.10.4.1	Intra-frequency absolute S-RSSI measurement accuracy requirements	730

9.10.4.2 Intra-frequency relative S-RSSI measurement accuracy requirements	730

9.11	NR Measurements	731

9.11.1	NR SS-RSRP Measurements	731

9.11.2	NR SS-RSRQ Measurements	731

9.11.3	NR SS-SINR Measurements	732

10	Measurements Performance Requirements for E-UTRAN	732

10.1	Received Interference Power	732

10.1.1	Absolute accuracy requirement	732

10.1.2	Relative accuracy requirement	733

10.1.3	Received Interference Power measurement report mapping	733

10.2	Angle of Arrival (AOA)	733

10.2.1	Range/mapping	733

10.3	Timing Advance (TADV)	733

10.3.1	Report mapping	733

11	ProSe Requirements in Any Cell Selection state	734

11.1	Introduction	734

11.2	UE Transmit Timing for ProSe in Any Cell Selection State	734

11.2.1	Introduction	734

11.2.2	ProSe UE transmission timing	734

11.3	Initiation/Cease of SLSS Transmissions	735

11.3.1	Introduction	735

11.3.2	Requirements	735

11.4	Measurements for ProSe in Any Cell Selection State	735

11.4.1	Introduction	735

11.4.2	Requirements	735

11.4.2.1	E-UTRA FDD	735

11.4.2.2	E-UTRA TDD	736

11.5	Selection / Reselection of ProSe Synchronization Reference	736

11.5.1	Introduction	736

11.5.2	Selection/Reselection to intra-frequency SyncRef UE	736

11.5.2.1	Introduction	736

11.5.2.2	Requirements	736

11.6	Void	737

11.7	Selection / Reselection of ProSe relay UE	737

11.7.1	Introduction	737

11.7.2	Selection / Reselection of intra-frequency ProSe relay UE	737

12	V2V Sidelink Communication Requirements for V2V Operation on Dedicated V2V Carrier	737

12.1	Introduction	737

12.2	Transmit Timing	738

12.2.1	GNSS as timing reference	738

12.3	Interruption	738

12.4	Reliability of GNSS signal	738

13	V2X Requirements	738

13.1	Introduction	738

13.2	UE Transmit Timing	739

13.2.1	Introduction	739

13.2.2	GNSS as synchronization reference source	739

13.2.3	Serving cell/PCell as synchronization reference source	739

13.2.4	SyncRef UE as synchronization reference source	739

13.3	Initiation/Cease of SLSS Transmissions	739

13.3.1	Introduction	739

13.3.1.1	Initiation/Cease of SLSS transmissions with Serving cell / PCell as synchronization reference source	739

13.3.1.2	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	740

13.3.1.3	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	740

13.4	Selection / Reselection of V2X Synchronization Reference Source	741

13.5	Autonomous Resource Selection/Reselection measurements	742

13.5.1	Introduction	742

13.5.2	PSSCH-RSRP measurements	742

13.5.3	S-RSSI measurements	742

13.6	Congestion Control measurements	742

13.7	Interruption	742

13.7.1	Interruptions to WAN due to V2X Sidelink Communication	742

13.7.2	V2X Sidelink Communication Dropping due to synchronization reference source change	742

13.7.3	Interruptions to WAN due to V2X Carrier Aggregation	743

13.8	Reliability of GNSS signal	743

13.9	Component Carrier Addition and Release Delay for V2X Sidelink Carrier Aggregation	743

13.10	Selection / Reselection of V2X Synchronization Reference Source for V2X Carrier Aggregation	744

Annex A (normative):	Test Cases	745

A.1	Purpose of annex	745

A.2	Requirement classification for statistical testing	745

A.2.1	Types of requirements in TS 36.133	745

A.2.1.1	Time and delay requirements on UE higher layer actions	745

A.2.1.2	Measurements of power levels, relative powers and time	745

A.2.1.3	Implementation requirements	746

A.2.1.4	Physical layer timing requirements	746

A.3	RRM test configurations	747

A.3.1	Reference Measurement Channels	747

A.3.1.1	PDSCH	747

A.3.1.1.1	FDD	747

A.3.1.1.2	TDD	751

A.3.1.1.3	FDD for UE category 0	754

A.3.1.1.4	HD-FDD for UE category 0	755

A.3.1.1.5	TDD for UE category 0	756

A.3.1.1.6	Frame Structure 3	757

A.3.1.2	PCFICH/PDCCH/PHICH	758

A.3.1.2.1	FDD	758

A.3.1.2.2	TDD	758

A.3.1.2.3	HD-FDD for UE category 0	759

A.3.1.2.4	FS 3	759

A.3.1.3	MPDCCH Reference Channels for Cat-M1 UEs	759

A.3.1.3.1	FDD in CEModeA	760

A.3.1.3.2	HD-FDD in CEModeA	760

A.3.1.3.3	TDD in CEModeA	761

A.3.1.3.4	FDD in CEModeB	761

A.3.1.3.5	HD-FDD in CEModeB	762

A.3.1.3.6	TDD in CEModeB	762

A.3.1.4	PDSCH Reference Channel for Cat-M1 UEs	763

A.3.1.4.1	FDD in CEModeA	763

A.3.1.4.2	HD-FDD in CEModeA	764

A.3.1.4.3	TDD in CEModeA	765

A.3.1.4.4	FDD in CEModeB	766

A.3.1.4.5	 HD-FDD in CEModeB	767

A.3.1.4.6	TDD in CEModeB	767

A.3.1.5	NPDSCH Reference Channel for UE category NB1	768

A.3.1.5.1	HD-FDD in-band operation	768

A.3.1.5.2	Void	769

A.3.1.5.3	HD-FDD standalone operation	769

A.3.1.5.4	Void	770

A.3.1.5.5	HD-FDD guard band operation	770

A.3.1.5.6	Void	771

A.3.1.5.7	TDD in-band operation	771

A.3.1.5.8	TDD standalone operation	771

A.3.1.5.9	TDD guard band operation	772

A.3.1.6	NPDCCH Reference Channel for UE category NB1	772

A.3.1.6.1	In-band operation	772

A.3.1.6.2	Void	773

A.3.1.6.3	Standalone operation	773

A.3.1.6.4	Void	773

A.3.1.6.5	Guard band operation	773

A.3.1.6.6	Void	774

A.3.2	OFDMA Channel Noise Generator (OCNG)	774

A.3.2.1	OCNG Patterns for FDD	774

A.3.2.1.1	OCNG FDD pattern 1: outer resource blocks allocation in 10 MHz	775

A.3.2.1.2	OCNG FDD pattern 2: full bandwidth allocation in 10 MHz	775

A.3.2.1.3	OCNG FDD pattern 3: outer resource blocks allocation in 1.4 MHz	776

A.3.2.1.4	OCNG FDD pattern 4: full bandwidth allocation in 1.4 MHz	776

A.3.2.1.5	OCNG FDD pattern 5: outer resource blocks allocation in 10 MHz (without MBSFN)	777

A.3.2.1.6	OCNG FDD pattern 6: full bandwidth allocation in 10 MHz (without MBSFN)	778

A.3.2.1.7	OCNG FDD pattern 7: full bandwidth allocation in 1.4 MHz (without MBSFN)	778

A.3.2.1.8	OCNG FDD pattern 8: outer resource blocks allocation in 10 MHz for MBSFN ABS	778

A.3.2.1.9	OCNG FDD pattern 9: full bandwidth allocation in 10 MHz for MBSFN ABS	779

A.3.2.1.10	OCNG FDD pattern 10: outer resource blocks allocation in 10 MHz with user data in every subframe (without MBSFN)	780

A.3.2.1.11	OCNG FDD pattern 11: outer resource blocks allocation in 20 MHz	780

A.3.2.1.12	OCNG FDD pattern 12: full bandwidth allocation in 20 MHz	781

A.3.2.1.13	OCNG FDD pattern 13: outer resource blocks allocation in 20 MHz (without MBSFN)	781

A.3.2.1.14	OCNG FDD pattern 14: full bandwidth allocation in 20 MHz (without MBSFN)	782

A.3.2.1.15	OCNG FDD pattern 15: outer resource blocks allocation in 5 MHz	782

A.3.2.1.16	OCNG FDD pattern 16: full bandwidth allocation in 5 MHz	783

A.3.2.1.17	OCNG FDD pattern 17: outer resource blocks allocation in 20 MHz with user data in every subframe (without MBSFN)	783

A.3.2.1.18	OCNG FDD pattern 18: outer resource blocks allocation in 5 MHz (without MBSFN)	784

A.3.2.1.19	OCNG FDD pattern 19: full bandwidth allocation in 5 MHz (without MBSFN)	784

A.3.2.1.20	OCNG FDD pattern 20: outer resource blocks allocation in 5 MHz with user data in every subframe (without MBSFN)	785

A.3.2.1.21	OCNG FDD pattern 21: Generic resource blocks allocation (without MBSFN)	785

A.3.2.1.22	OCNG FDD pattern 22: Generic resource blocks allocation in 5MHz (without MBSFN)	786

A.3.2.2	OCNG Patterns for TDD	786

A.3.2.2.1	OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	787

A.3.2.2.2	OCNG TDD pattern 2: full bandwidth allocation in 10 MHz	787

A.3.2.2.3	OCNG TDD pattern 3: outer resource blocks allocation in 1.4 MHz	788

A.3.2.2.4	OCNG TDD pattern 4: full bandwidth allocation in 1.4 MHz	788

A.3.2.2.5	OCNG TDD pattern 5: outer resource blocks allocation in 10 MHz for MBSFN ABS	789

A.3.2.2.6	OCNG TDD pattern 6: full bandwidth allocation in 10 MHz for MBSFN ABS	790

A.3.2.2.7	OCNG TDD pattern 7: outer resource blocks allocation in 20 MHz	791

A.3.2.2.8	OCNG TDD pattern 8: full bandwidth allocation in 20 MHz	792

A.3.2.2.9	OCNG TDD pattern 9: outer resource blocks allocation in 5 MHz	792

A.3.2.2.10	OCNG TDD pattern 10: full bandwidth allocation in 5 MHz	793

A.3.2.2.11	OCNG TDD pattern 11: Generic resource blocks allocation (without MBSFN)	793

A.3.2.3	OCNG Patterns for Narrowband IoT	794

A.3.2.3.1	Narrowband IoT OCNG FDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	795

A.3.2.3.2	Narrowband IoT OCNG FDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	796

A.3.2.3.3	Narrowband IoT OCNG FDD pattern 3: standalone NB-IoT	796

A.3.2.3.4	Narrowband IoT OCNG FDD pattern 4: In-band NB-IoT in 5 MHz EUTRAN cell	797

A.3.2.3.5	Narrowband IoT OCNG FDD pattern 5: guard band NB-IoT in 5 MHz EUTRAN cell	798

A.3.2.3.6	Narrowband IoT OCNG TDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	799

A.3.2.3.7	Narrowband IoT OCNG TDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	801

A.3.2.3.8	Narrowband IoT OCNG TDD pattern 3: standalone NB-IoT	801

A.3.2.4	OCNG Patterns for V2X sidelink	802

A.3.2.4.1	V2X sidelink OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	802

A.3.2.4.2	V2X sidelink OCNG TDD pattern 2: outer resource blocks allocation in 10 MHz	803

A.3.3	Reference DRX Configurations	803

A.3.4	ABS Transmission Configurations	804

A.3.4.1	Non-MBSFN ABS Transmission Configurations	804

A.3.4.1.1	Non-MBSFN ABS Transmission, 1x2 antenna with PBCH	804

A.3.4.1.2	Non-MBSFN ABS Transmission, 2x2 antenna without PBCH	804

A.3.4.2	MBSFN ABS Transmission Configurations	805

A.3.4.2.1	MBSFN ABS Transmission, 1x2 antenna	805

A.3.4.2.2	MBSFN ABS Transmission, 2x2 antenna	806

A.3.5	Impact of Reference Sensitivity Degradation with Carrier Aggregation on Test Cases	806

A.3.5.1	Impact of Reference Sensitivity Degradation due to Insertion Loss	806

A.3.6	Carrier Aggregation Test Cases with Different Channel Bandwidth Combinations	807

A.3.6.1	Introduction	807

A.3.7	Test Cases with Different Channel Bandwidths	807

A.3.7.1	Introduction	807

A.3.7.2	Principle of testing	807

A.3.8	Antenna Configuration	807

A.3.8.1	Antenna connection for 4 Rx capable UEs	807

A.3.8.1.1 Introduction	807

A.3.8.1.2 Principle of testing	807

A.3.8.1.2.1 Single carrier tests	807

A.3.8.1.2.2	Carrier aggregation and Dual connectivity tests	808

A.3.8.1.2.3	Antenna connection for bands where 2RX is supported	809

A.3.8.1.2.4	Antenna connection for bands where 4RX is supported	809

A.3.8.2	Antenna connection for 8 Rx capable UEs	809

A.3.8.2.1	Introduction	809

A.3.8.2.2	Principle of testing	809

A.3.8.2.2.1	Single carrier tests	809

A.3.8.2.2.2	Carrier aggregation and Dual connectivity tests	810

A.3.8.2.2.3	Antenna connection for bands where 2RX is supported	810

A.3.8.2.2.4	Antenna connection for bands where 4RX is supported	810

A.3.8.2.2.5	Antenna connection for bands where 8RX is supported	810

A.3.9	Carrier Aggregation Test Cases with Different Duplex Modes	810

A.3.9.1	Introduction	810

A.3.9.2	Principle of testing	810

A.3.10	Carrier Aggregation Test Cases with Different CA Configurations	811

A.3.10.1	Introduction	811

A.3.10.2	Principle of testing	811

A.3.11	Test Cases for Synchronous and Asynchronous Dual Connectivity	811

A.3.11.1	Introduction	811

A.3.11.2	Principle of Testing	811

A.3.12	Proximity-based Services	811

A.3.12.1	Introduction	811

A.3.12.2	Reference DRX configurations for ProSe tests	811

A.3.12.3	Test Cases with Different Channel Bandwidths	812

A.3.12.3.1	Introduction	812

A.3.12.3.2	Principle of testing	812

A.3.12.4	Reference resource pool configurations for ProSe Direct Discovery	812

A.3.12.5	Reference resource pool configurations for ProSe Direct Communication	818

A.3.12.6	Reference Measurement Channels for ProSe Direct Discovery	820

A.3.12.6.1	FDD	820

A.3.12.7	Reference measurement channels for ProSe Direct Communication	820

A.3.12.7.1	FDD	820

A.3.12.8	ProSe Receive Traffic Generator	821

A.3.12.8.1	ProSe Direct Communication Receive Traffic Generator for FDD	821

A.3.12.8.2	ProSe Direct Discovery Receive Traffic Generator for FDD	821

A.3.13	Time Offset between Cells	822

A.3.13.1	Introduction	822

A.3.13.2	Definition	822

A.3.14	Carrier Aggregation under operation with Frame Structure 3 Test Cases with Different Duplex Modes	822

A.3.14.1	Introduction	822

A.3.14.2	Principle of testing	822

A.3.15	Dual connectivity test cases with different combination of duplex mode	822

A.3.15.1	Introduction	822

A.3.15.2	Principle of testing	822

A.3.16	Reference PRACH Configurations	823

A.3.17	Listen before talk model	823

A.3.17.1	Introduction	823

A.3.17.2	Definition	823

A.3.18	Reference NPRACH Configurations	824

A.3.19	Dual connectivity test cases with different bandwidth combinations	825

A.3.19.1	Introduction	825

A.3.19.2	Principle of testing	825

A.3.20	Category M1 UE Test Cases	826

A.3.20.1	Introduction	826

A.3.20.2	Principle of Cat-M1 UE Testing	826

A.3.20.3	Principle of Cat-M1 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	827

A.3.21	V2V Sidelink Communication on Dedicated V2V Carrier	828

A.3.21.1	Introduction	828

A.3.21.2	Reference resource pool configurations for V2V Sidelink Communication	828

A.3.21.3	Reference measurement channels for V2V Sidelink Communication	829

A.3.22	Category 1bis UE Test Cases	830

A.3.22.1	Introduction	830

A.3.22.2	Principle of Category 1bis UE Testing	830

A.3.23	Category NB2 UE Test Cases	833

A.3.23.1	Introduction	833

A.3.23.2	Principle of Category NB2 UE Testing	833

A.3.24	V2X sidelink communication	835

A.3.24.1	Introduction	835

A.3.24.2	Reference resource pool configurations for V2X Sidelink Communication	836

A.3.24.3	Reference measurement channels for V2X Sidelink Communication	839

A.3.25	Category M2 UE Test Cases	840

A.3.25.1	Introduction	840

A.3.25.2	Principle of Cat-M2 UE Testing	840

A.3.25.3	Principle of Cat-M2 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	841

A.3.26	sTTI and processing time reduction test cases with different sTTI/processing time reduction scheme	842

A.3.26.1	Introduction	842

A.3.26.2	Principle of testing	842

A.3.27	LTE INACTIVE Cell Re-selection Test Cases	842

A.3.27.1	Introduction	842

A.3.27.2	Principle of INACTIVE cell re-selection Testing	842

A.4	E-UTRAN RRC_IDLE state	842

A.4.2	Cell Re-Selection	842

A.4.2.1	E-UTRAN FDD – FDD Intra frequency case	842

A.4.2.1.1	Test Purpose and Environment	842

A.4.2.1.2	Test Requirements	844

A.4.2.2	E-UTRAN TDD – TDD Intra frequency case	845

A.4.2.2.1	Test Purpose and Environment	845

A.4.2.2.2	Test Requirements	846

A.4.2.3	E-UTRAN FDD – FDD Inter frequency case	847

A.4.2.3.1	Test Purpose and Environment	847

A.4.2.3.2	Test Requirements	848

A.4.2.4	E-UTRAN FDD – TDD Inter frequency case	849

A.4.2.4.1	Test Purpose and Environment	849

A.4.2.4.2	Test Requirements	850

A.4.2.5	E-UTRAN TDD – FDD Inter frequency case	851

A.4.2.5.1	Test Purpose and Environment	851

A.4.2.5.2	Test Requirements	852

A.4.2.6	E-UTRAN TDD – TDD: Inter frequency case	853

A.4.2.6.1	Test Purpose and Environment	853

A.4.2.6.2	Test Requirements	854

A.4.2.7	E-UTRAN FDD – FDD Inter frequency case in the existence of non-allowed CSG cell	855

A.4.2.7.1	Test Purpose and Environment	855

A.4.2.7.2	Test Requirements	856

A.4.2.8	E-UTRAN TDD – TDD Inter frequency case in the existence of non-allowed CSG cell	857

A.4.2.8.1	Test Purpose and Environment	857

A.4.2.8.2	Test Requirements	858

A.4.2.9	E-UTRAN FDD – FDD Intra frequency case for 5MHz bandwidth	859

A.4.2.9.1	Test Purpose and Environment	859

A.4.2.9.2	Test Requirements	859

A.4.2.10	E-UTRAN FDD – FDD reselection using an increased number of carriers	859

A.4.2.10.1	Test Purpose and Environment	859

A.4.2.10.2	Test Requirements	863

A.4.2.11	E-UTRAN TDD – TDD reselection using an increased number of carriers	863

A.4.2.11.1	Test Purpose and Environment	863

A.4.2.11.2	Test Requirements	867

A.4.2.12	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage	867

A.4.2.12.1	Test Purpose and Environment	867

A.4.2.12.2	Test Requirements	869

A.4.2.13	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage	870

A.4.2.13.1	Test Purpose and Environment	870

A.4.2.13.2	Test Requirements	871

A.4.2.14	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	872

A.4.2.14.1	Test Purpose and Environment	872

A.4.2.14.2	Test Requirements	873

A.4.2.15 E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	874

A.4.2.15.1	Test Purpose and Environment	874

A.4.2.15.2	Test Requirements	875

A.4.2.16 E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	876

A.4.2.16.1	Test Purpose and Environment	876

A.4.2.16.2	Test Requirements	877

A.4.2.17 E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in enhanced coverage	878

A.4.2.17.1	Test Purpose and Environment	878

A.4.2.17.2	Test Requirements	879

A.4.2.18 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	880

A.4.2.18.1	Test Purpose and Environment	880

A.4.2.18.2	Test Requirements	882

A.4.2.19	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	883

A.4.2.19.1	Test Purpose and Environment	883

A.4.2.19.2	Test Requirements	885

A.4.2.20	E-UTRAN FDD – FDD Intra frequency case for UE Category 1bis	886

A.4.2.20.1	Test Purpose and Environment	886

A.4.2.20.2	Test Requirements	887

A.4.2.21	E-UTRAN TDD – TDD Intra frequency case for UE Category 1bis	888

A.4.2.21.1	Test Purpose and Environment	888

A.4.2.21.2	Test Requirements	889

A.4.2.22	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	890

A.4.2.22.1	Test Purpose and Environment	890

A.4.2.22.2	Test Requirements	891

A.4.2.23	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	892

A.4.2.23.1	Test Purpose and Environment	892

A.4.2.23.2	Test Requirements	893

A.4.2.24	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	894

A.4.2.24.1	Test Purpose and Environment	894

A.4.2.24.2	Test Requirements	896

A.4.2.25	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	897

A.4.2.25.1	Test Purpose and Environment	897

A.4.2.25.2	Test Requirements	898

A.4.2.26	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage	899

A.4.2.26.1	Test Purpose and Environment	899

A.4.2.26.2	Test Requirements	900

A.4.2.27	E-UTRAN TDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	901

A.4.2.27.1	Test Purpose and Environment	901

A.4.2.27.2	Test Requirements	902

A.4.2.28	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	903

A.4.2.28.1	Test Purpose and Environment	903

A.4.2.28.2	Test Requirements	904

A.4.2.29	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	905

A.4.2.29.1	Test Purpose and Environment	905

A.4.2.29.2	Test Requirements	906

A.4.2.30	E-UTRAN TDD Inter frequency case for Cat-M1 UE in enhanced coverage	907

A.4.2.30.1	Test Purpose and Environment	907

A.4.2.30.2	Test Requirements	908

A.4.2.31	E-UTRAN FDD – FDD Inter frequency case for UE Category 1bis	909

A.4.2.31.1	Test Purpose and Environment	909

A.4.2.31.2	Test Requirements	910

A.4.2.32	E-UTRAN FDD – TDD Inter frequency case for UE Category 1bis	911

A.4.2.32.1	Test Purpose and Environment	911

A.4.2.32.2	Test Requirements	912

A.4.2.33	E-UTRAN TDD – FDD Inter frequency case for UE Category 1bis	913

A.4.2.33.1	Test Purpose and Environment	913

A.4.2.33.2	Test Requirements	914

A.4.2.34	E-UTRAN TDD – TDD: Inter frequency case for UE Category 1bis	915

A.4.2.34.1	Test Purpose and Environment	915

A.4.2.34.2	Test Requirements	916

A.4.2.35	E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	917

A.4.2.35.1	Test Purpose and Environment	917

A.4.2.35.2	Test Requirements	919

A.4.2.36	E-UTRAN TDD – TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	920

A.4.2.36.1	Test Purpose and Environment	920

A.4.2.36.2	Test Requirements	922

A.4.2.37	E-UTRAN TDD – TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	923

A.4.2.37.1	Test Purpose and Environment	923

A.4.2.37.2	Test Requirements	925

A.4.2.38	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation	926

A.4.2.38.1	Test Purpose and Environment	926

A.4.2.38.2	Test Requirements	928

A.4.3	E-UTRAN to UTRAN Cell Re-Selection	929

A.4.3.1	E-UTRAN FDD – UTRAN FDD:	929

A.4.3.1.1	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of higher priority	929

A.4.3.1.1.1	Test Purpose and Environment	929

A.4.3.1.1.2	Test Requirements	931

A.4.3.1.2	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority	931

A.4.3.1.2.1	Test Purpose and Environment	931

A.4.3.1.2.2	Test Requirements	934

A.4.3.1.3	EUTRA FDD-UTRA FDD cell reselection in fading propagation conditions: UTRA FDD is of lower priority	934

A.4.3.1.3.1	Test Purpose and Environment	934

A.4.3.1.3.2	Test Requirements	937

A.4.3.1.4	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority for 5MHz bandwidth	937

A.4.3.1.4.1	Test Purpose and Environment	937

A.4.3.1.4.2	Test Requirements	938

A.4.3.1.5	Idle mode FDD to UTRA FDD interRAT reselection	938

A.4.3.1.5.1	Test Purpose and Environment	938

A.4.3.1.5.2	Test Requirements	941

A.4.3.2	E-UTRAN FDD – UTRAN TDD:	942

A.4.3.2.1	Test Purpose and Environment	942

A.4.3.2.1.1	Void	942

A.4.3.2.1.2	1.28Mcps TDD option	942

A.4.3.2.1.3	Void	944

A.4.3.2.2	Test Requirements	944

A.4.3.2.2.1	1.28Mcps TDD option	944

A.4.3.2A	E-UTRA FDD to UTRA TDD cell re-selection for IncMon	944

A.4.3.2A.1	Test Purpose and Environment	944

A.4.3.2A.2	Test Requirements	948

A.4.3.3	E-UTRAN TDD – UTRAN FDD:	948

A.4.3.3.1	Test Purpose and Environment	948

A.4.3.3.2	Test Requirements	951

A.4.3.3A	Idle mode TDD to UTRA FDD interRAT reselection	951

A.4.3.3A.1	Test Purpose and Environment	951

A.4.3.3A.2	Test Requirements	956

A.4.3.4	E-UTRAN TDD – UTRAN TDD:	957

A.4.3.4.1	E-UTRA to UTRA TDD cell re-selection: UTRA is of higher priority	957

A.4.3.4.1.1	Test Purpose and Environment	957

A.4.3.4.1.2	Test Requirements	959

A.4.3.4.2	E-UTRA to UTRA TDD cell re-selection: UTRA is of lower priority	959

A.4.3.4.2.1	Test Purpose and Environment	959

A.4.3.4.2.2	Test Requirements	961

A.4.3.4.3	EUTRA TDD-UTRA TDD cell reselection in fading propagation conditions: UTRA TDD is of lower priority	961

A.4.3.4.3.1	Test Purpose and Environment	961

A.4.3.4.3.2	Test Requirements	964

A.4.3.4.4	E-UTRA TDD to UTRA TDD cell re-selection for IncMon	964

A.4.3.4.4.1	Test Purpose and Environment	964

A.4.3.4.4.2	Test Requirements	968

A.4.4	E-UTRAN to GSM Cell Re-Selection	968

A.4.4.1	E-UTRAN FDD – GSM:	968

A.4.4.1.1	Test Purpose and Environment	968

A.4.4.1.2	Test Requirements	970

A.4.4.2	E-UTRAN TDD – GSM:	970

A.4.4.2.1	Test Purpose and Environment	970

A.4.4.2.2	Test Requirements	972

A.4.5	E-UTRAN to HRPD Cell Re-Selection	973

A.4.5.1	E-UTRAN FDD – HRPD	973

A.4.5.1.1	E-UTRAN FDD – HRPD Cell Reselection: HRPD is of Lower Priority	973

A.4.5.1.1.1	Test Purpose and Environment	973

A.4.5.1.1.2	Test Requirements	975

A.4.5.2	E-UTRAN TDD – HRPD	975

A.4.5.2.1	E-UTRAN TDD – HRPD Cell Reselection: HRPD is of Lower Priority	975

A.4.5.2.1.1	Test Purpose and Environment	975

A.4.5.2.1.2	Test Requirements	978

A.4.6	E-UTRAN to cdma2000 1X Cell Re-Selection	978

A.4.6.1	E-UTRAN FDD – cdma2000 1X	978

A.4.6.1.1	E-UTRAN FDD – cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	978

A.4.6.1.1.1	Test Purpose and Environment	978

A.4.6.1.1.2	Test Requirements	981

A.4.6.2	E-UTRAN TDD – cdma2000 1X	981

A.4.6.2.1	E-UTRAN TDD –cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	981

A.4.6.2.1.1	Test Purpose and Environment	981

A.4.6.2.1.2	Test Requirements	984

A.4.7	Idle State Positioning Measurement for UE category NB1	984

A.4.7.1	HD – FDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	984

A.4.7.1.1	Test Purpose and Environment	984

A.4.7.1.2	Test Requirements	988

A.4.7.2	HD – FDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	989

A.4.7.2.1	Test Purpose and Environment	989

A.4.7.4.2	Test Requirements	992

A.4.7.3	TDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	993

A.4.7.3.1	Test Purpose and Environment	993

A.4.7.3.2	Test Requirements	996

A.4.7.4	TDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	997

A.4.7.4.1	Test Purpose and Environment	997

A.4.7.4.2	Test Requirements	1000

A.5	E-UTRAN RRC CONNECTED Mode Mobility	1001

A.5.1	E-UTRAN Handover	1001

A.5.1.1	E-UTRAN FDD - FDD Intra frequency handover	1001

A.5.1.1.1	Test Purpose and Environment	1001

A.5.1.1.2	Test Requirements	1003

A.5.1.2	E-UTRAN TDD - TDD Intra frequency handover	1003

A.5.1.2.1	Test Purpose and Environment	1003

A.5.1.2.2	Test Requirements	1005

A.5.1.3	E-UTRAN FDD – FDD Inter frequency handover	1005

A.5.1.3.1	Test Purpose and Environment	1005

A.5.1.3.2	Test Requirements	1007

A.5.1.4	E-UTRAN TDD – TDD Inter frequency handover	1007

A.5.1.4.1	Test Purpose and Environment	1007

A.5.1.4.2	Test Requirements	1009

A.5.1.5	E-UTRAN FDD – FDD Inter frequency handover: unknown target cell	1009

A.5.1.5.1	Test Purpose and Environment	1009

A.5.1.5.2	Test Requirements	1011

A.5.1.6 	E-UTRAN TDD – TDD Inter frequency handover; unknown Target Cell	1011

A.5.1.6.1	Test Purpose and Environment	1011

A.5.1.6.2	Test Requirements	1013

A.5.1.7	E-UTRAN FDD – TDD Inter frequency handover	1013

A.5.1.7.1	Test Purpose and Environment	1013

A.5.1.7.2	Test Requirements	1016

A.5.1.8	E-UTRAN TDD – FDD Inter frequency handover	1016

A.5.1.8.1	Test Purpose and Environment	1016

A.5.1.8.2	Test Requirements	1019

A.5.1.9	E-UTRAN FDD - FDD Intra frequency handover for 5MHz bandwidth	1019

A.5.1.9.1	Test Purpose and Environment	1019

A.5.1.9.2	Test Requirements	1020

A.5.1.10	E-UTRAN FDD - FDD Intra frequency handover for UE category 0	1020

A.5.1.10.1	Test Purpose and Environment	1020

A.5.1.10.2	Test Requirements	1022

A.5.1.11	E-UTRAN HD - FDD Intra frequency handover for UE category 0	1022

A.5.1.11.1	Test Purpose and Environment	1022

A.5.1.11.2	Test Requirements	1024

A.5.1.12	E-UTRAN TDD - TDD Intra frequency handover for UE category 0	1024

A.5.1.12.1	Test Purpose and Environment	1024

A.5.1.12.2	Test Requirements	1026

A.5.1.13	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1026

A.5.1.13.1	Test Purpose and Environment	1026

A.5.1.13.2	Test Requirements	1028

A.5.1.14	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1029

A.5.1.14.1	Test Purpose and Environment	1029

A.5.1.14.2	Test Requirements	1030

A.5.1.15	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA	1031

A.5.1.15.1	Test Purpose and Environment	1031

A.5.1.15.2	Test Requirements	1032

A.5.1.16	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1033

A.5.1.16.1	Test Purpose and Environment	1033

A.5.1.16.2	Test Requirements	1034

A.5.1.17	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1035

A.5.1.17.1	Test Purpose and Environment	1035

A.5.1.17.2	Test Requirements	1036

A.5.1.18	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB	1037

A.5.1.18.1	Test Purpose and Environment	1037

A.5.1.18.2	Test Requirements	1038

A.5.1.19	E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis	1039

A.5.1.19.1	Test Purpose and Environment	1039

A.5.1.19.2	Test Requirements	1040

A.5.1.20	E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis	1041

A.5.1.20.1	Test Purpose and Environment	1041

A.5.1.20.2	Test Requirements	1042

A.5.1.21	E-UTRAN FDD - FDD Intra frequency RACH-less handover	1043

A.5.1.21.1	Test Purpose and Environment	1043

A.5.1.21.2	Test Requirements	1044

A.5.1.22	E-UTRAN TDD - TDD Intra frequency RACH-less handover	1044

A.5.1.22.1	Test Purpose and Environment	1044

A.5.1.22.2	Test Requirements	1046

A.5.1.23	E-UTRAN FDD – FDD Inter frequency RACH-less handover	1046

A.5.1.23.1	Test Purpose and Environment	1046

A.5.1.23.2	Test Requirements	1048

A.5.1.24	E-UTRAN TDD – TDD Inter frequency RACH-less handover	1048

A.5.1.24.1	Test Purpose and Environment	1048

A.5.1.24.2	Test Requirements	1050

A.5.1.25	E-UTRAN FDD - FDD Intra frequency make-before-break handover	1050

A.5.1.25.1	Test Purpose and Environment	1050

A.5.1.25.2	Test Requirements	1052

A.5.1.26	E-UTRAN TDD - TDD Intra frequency make-before-break handover	1052

A.5.1.26.1	Test Purpose and Environment	1052

A.5.1.26.2	Test Requirements	1054

A.5.1.27	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeA	1055

A.5.1.27.1	Test Purpose and Environment	1055

A.5.1.27.2	Test Requirements	1056

A.5.1.28	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeA	1057

A.5.1.28.1	Test Purpose and Environment	1057

A.5.1.28.2	Test Requirements	1058

A.5.1.29	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeA	1059

A.5.1.29.1	Test Purpose and Environment	1059

A.5.1.29.2	Test Requirements	1060

A.5.1.30	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeB	1061

A.5.1.30.1	Test Purpose and Environment	1061

A.5.1.30.2	Test Requirements	1062

A.5.1.31	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeB	1063

A.5.1.31.1	Test Purpose and Environment	1063

A.5.1.31.2	Test Requirements	1064

A.5.1.32	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeB	1065

A.5.1.32.1	Test Purpose and Environment	1065

A.5.1.32.2	Test Requirements	1066

A.5.1.33	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1067

A.5.1.33.1	Test Purpose and Environment	1067

A.5.1.13.2	Test Requirements	1068

A.5.1.34	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1069

A.5.1.34.1	Test Purpose and Environment	1069

A.5.1.34.2	Test Requirements	1070

A.5.1.35	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1071

A.5.1.35.1	Test Purpose and Environment	1071

A.5.1.35.2	Test Requirements	1072

A.5.1.36	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1073

A.5.1.36.1	Test Purpose and Environment	1073

A.5.1.36.2	Test Requirements	1074

A.5.1.37	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1075

A.5.1.37.1	Test Purpose and Environment	1075

A.5.1.37.2	Test Requirements	1076

A.5.1.38	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1077

A.5.1.38.1	Test Purpose and Environment	1077

A.5.1.38.2	Test Requirements	1078

A.5.1.39	E-UTRAN FDD - FDD Intra frequency handover with direct SCell activation	1079

A.5.1.39.1	Test Purpose and Environment	1079

A.5.1.39.2	Test Requirements	1080

A.5.1.40	E-UTRAN TDD - TDD Intra frequency handover with direct SCell activation	1081

A.5.1.40.1	Test Purpose and Environment	1081

A.5.1.40.2	Test Requirements	1082

A.5.2	E-UTRAN Handover to other RATs	1083

A.5.2.1	E-UTRAN FDD – UTRAN FDD Handover	1083

A.5.2.1.1	Test Purpose and Environment	1083

A.5.2.1.2	Test Requirements	1085

A.5.2.2	E-UTRAN TDD - UTRAN FDD Handover	1086

A.5.2.2.1	Test Purpose and Environment	1086

A.5.2.2.2	Test Requirements	1089

A.5.2.3 E-UTRAN FDD- GSM Handover	1089

A.5.2.3.1	Test Purpose and Environment	1089

A.5.2.3.2	Test Requirements	1090

A.5.2.4	E-UTRAN TDD - UTRAN TDD Handover	1091

A.5.2.4.1	Test Purpose and Environment	1091

A.5.2.4.1.1	Void	1091

A.5.2.4.1.2	1.28 Mcps TDD option	1091

A.5.2.4.1.3	Void	1093

A.5.2.4.2	Test Requirements	1093

A.5.2.4.2.1	Void	1093

A.5.2.4.2.2	1.28 Mcps TDD option	1093

A.5.2.4.2.3	Void	1093

A.5.2.5	E-UTRAN FDD – UTRAN TDD Handover	1093

A.5.2.5.1	Test Purpose and Environment	1093

A.5.2.5.1.3	Void	1096

A.5.2.5.2	Test Requirements	1096

A.5.2.5.2.1	Void	1096

A.5.2.5.2.2	1.28 Mcps TDD option	1096

A.5.2.5.2.3	Void	1096

A.5.2.6	E-UTRAN TDD - GSM Handover	1096

A.5.2.6.1	Test Purpose and Environment	1096

A.5.2.6.2	Test Requirements	1098

A.5.2.7	E-UTRAN FDD – UTRAN FDD Handover; Unknown Target Cell	1099

A.5.2.7.1	Test Purpose and Environment	1099

A.5.2.7.2	Test Requirements	1101

A.5.2.8	E-UTRAN FDD - GSM Handover; Unknown Target Cell	1101

A.5.2.8.1	Test Purpose and Environment	1101

A.5.2.8.2	Test Requirements	1102

A.5.2.9	E-UTRAN TDD - GSM Handover; Unknown Target Cell	1103

A.5.2.9.1	Test Purpose and Environment	1103

A.5.2.9.2	Test Requirements	1104

A.5.2.10	E-UTRAN TDD to UTRAN TDD handover: unknown target cell	1105

A.5.2.10.1	Test Purpose and Environment	1105

A.5.2.10.2	Test Requirements	1107

A.5.2.10A	E-UTRAN FDD – UTRAN FDD Multicarrier Handover with two target cells	1107

A.5.2.10A.1	Test Purpose and Environment	1107

A.5.2.10A.2	Test Requirements	1110

A.5.2.10B	E-UTRAN TDD – UTRAN FDD Multicarrier Handover with two target cells	1110

A.5.2.10B.1	Test Purpose and Environment	1110

A.5.2.10B.2	Test Requirements	1113

A.5.2.11	E-UTRAN FDD – UTRAN FDD Handover for 5MHz Bandwidth	1113

A.5.2.11.1	Test Purpose and Environment	1113

A.5.2.11.2	Test Requirements	1113

A.5.3	E-UTRAN Handover to Non-3GPP RATs	1114

A.5.3.1	E-UTRAN FDD – HRPD Handover	1114

A.5.3.1.1	Test Purpose and Environment	1114

A.5.3.1.2	Test Requirements	1116

A.5.3.2	E-UTRAN FDD – cdma2000 1X Handover	1116

A.5.3.2.1	Test Purpose and Environment	1116

A.5.3.2.2	Test Requirements	1119

A.5.3.3	E-UTRAN FDD – HRPD Handover; Unknown Target Cell	1119

A.5.3.3.1	Test Purpose and Environment	1119

A.5.3.3.2	Test Requirements	1121

A.5.3.4	E-UTRAN FDD – cdma2000 1X Handover; Unknown Target cell	1122

A.5.3.4.1	Test Purpose and Environment	1122

A.5.3.4.2	Test Requirements	1123

A.5.3.5	E-UTRAN TDD – HRPD Handover	1124

A.5.3.5.1	Test Purpose and Environment	1124

A.5.3.5.2	Test Requirements	1126

A.5.3.6	E-UTRAN TDD – cdma2000 1X Handover	1127

A.5.3.6.1	Test Purpose and Environment	1127

A.5.3.6.2	Test Requirements	1130

A.6	RRC Connection Control	1130

A.6.1	RRC Re-establishment	1130

A.6.1.1	E-UTRAN FDD Intra-frequency RRC Re-establishment	1130

A.6.1.1.1	Test Purpose and Environment	1130

A.6.1.1.2	Test Requirements	1131

A.6.1.2	E-UTRAN FDD Inter-frequency RRC Re-establishment	1132

A.6.1.2.1	Test Purpose and Environment	1132

A.6.1.2.2	Test Requirements	1133

A.6.1.3	E-UTRAN TDD Intra-frequency RRC Re-establishment	1134

A.6.1.3.1	Test Purpose and Environment	1134

A.6.1.3.2	Test Requirements	1135

A.6.1.4	E-UTRAN TDD Inter-frequency RRC Re-establishment	1136

A.6.1.4.1	Test Purpose and Environment	1136

A.6.1.4.2	Test Requirements	1137

A.6.1.5	E-UTRAN FDD Intra-frequency RRC Re-establishment for 5MHz bandwidth	1138

A.6.1.5.1	Test Purpose and Environment	1138

A.6.1.5.2	Test Requirements	1138

A.6.1.6	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for UE category 0	1138

A.6.1.6.1	Test Purpose and Environment	1138

A.6.1.6.2	Test Requirements	1140

A.6.1.7	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for UE category 0	1141

A.6.1.7.1	Test Purpose and Environment	1141

A.6.1.7.2	Test Requirements	1142

A.6.1.8	E-UTRAN TDD Intra-frequency RRC Re-establishment for UE category 0	1143

A.6.1.8.1	Test Purpose and Environment	1143

A.6.1.8.2	Test Requirements	1144

A.6.1.9	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1145

A.6.1.9.1	Test Purpose and Environment	1145

A.6.1.9.2	Test Requirements	1146

A.6.1.10	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1147

A.6.1.10.1	Test Purpose and Environment	1147

A.6.1.10.2	Test Requirements	1148

A.6.1.11	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1149

A.6.1.11.1	Test Purpose and Environment	1149

A.6.1.11.2	Test Requirements	1150

A.6.1.12	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1151

A.6.1.12.1	Test Purpose and Environment	1151

A.6.1.12.2	Test Requirements	1152

A.6.1.13	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1153

A.6.1.13.1	Test Purpose and Environment	1153

A.6.1.13.2	Test Requirements	1154

A.6.1.14	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1155

A.6.1.14.1	Test Purpose and Environment	1155

A.6.1.14.2	Test Requirements	1156

A.6.1.15	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1157

A.6.1.15.1	Test Purpose and Environment	1157

A.6.1.15.2	Test Requirements	1159

A.6.1.16	HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1160

A.6.1.16.1	Test Purpose and Environment	1160

A.6.1.16.2	Test Requirements	1162

A.6.1.17	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1163

A.6.1.17.1	Test Purpose and Environment	1163

A.6.1.17.2	Test Requirements	1164

A.6.1.18	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1165

A.6.1.18.1	Test Purpose and Environment	1165

A.6.1.18.2	Test Requirements	1166

A.6.1.19	E-UTRAN TDD-TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1167

A.6.1.19.1	Test Purpose and Environment	1167

A.6.1.19.2	Test Requirements	1168

A.6.1.20	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1169

A.6.1.20.1	Test Purpose and Environment	1169

A.6.1.20.2	Test Requirements	1170

A.6.1.21	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1171

A.6.1.21.1	Test Purpose and Environment	1171

A.6.1.21.2	Test Requirements	1172

A.6.1.22	E-UTRAN TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1173

A.6.1.22.1	Test Purpose and Environment	1173

A.6.1.22.2	Test Requirements	1174

A.6.1.23	E-UTRAN TDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1175

A.6.1.23.1	Test Purpose and Environment	1175

A.6.1.23.2	Test Requirements	1177

A.6.1.24	E-UTRAN TDD - TDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1178

A.6.1.24.1	Test Purpose and Environment	1178

A.6.1.24.2	Test Requirements	1180

A.6.2	Random Access	1181

A.6.2.1	E-UTRAN FDD – Contention Based Random Access Test	1181

A.6.2.1.1	Test Purpose and Environment	1181

A.6.2.1.2	Test Requirements	1182

A.6.2.1.2.1	Random Access Response Reception	1182

A.6.2.1.2.2	No Random Access Response Reception	1182

A.6.2.1.2.3	Receiving a NACK on msg3	1182

A.6.2.1.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1183

A.6.2.1.2.5	Reception of a Correct Message over Temporary C-RNTI	1183

A.6.2.1.2.6	Contention Resolution Timer expiry	1183

A.6.2.2	E-UTRAN FDD – Non-Contention Based Random Access Test	1183

A.6.2.2.1	Test Purpose and Environment	1183

A.6.2.2.2	Test Requirements	1184

A.6.2.2.2.1	Random Access Response Reception	1185

A.6.2.2.2.2	No Random Access Response Reception	1185

A.6.2.3	E-UTRAN TDD – Contention Based Random Access Test	1185

A.6.2.3.1	Test Purpose and Environment	1185

A.6.2.3.2	Test Requirements	1187

A.6.2.3.2.1	Random Access Response Reception	1187

A.6.2.3.2.2	No Random Access Response reception	1187

A.6.2.3.2.3	Receiving a NACK on msg3	1187

A.6.2.3.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1188

A.6.2.3.2.5	Reception of a Correct Message over Temporary C-RNTI	1188

A.6.2.3.2.6	Contention Resolution Timer expiry	1188

A.6.2.4	E-UTRAN TDD – Non-Contention Based Random Access Test	1188

A.6.2.4.1	Test Purpose and Environment	1188

A.6.2.4.2	Test Requirements	1190

A.6.2.4.2.1	Random Access Response Reception	1190

A.6.2.4.2.2	No Random Access Response Reception	1190

A.6.2.5	E-UTRAN FDD – Contention Based Random Access Test for 5MHz bandwidth	1190

A.6.2.5.1	Test Purpose and Environment	1190

A.6.2.5.2	Test Requirements	1191

A.6.2.6	E-UTRAN FDD – Non-contention Based Random Access Test for 5MHz bandwidth	1191

A.6.2.6.1	Test Purpose and Environment	1191

A.6.2.6.2	Test Requirements	1191

A.6.2.7	E-UTRAN FDD – Non-Contention Based Random Access Test For SCell	1192

A.6.2.7.1	Test Purpose and Environment	1192

A.6.2.7.2	Test Requirements	1194

A.6.2.7.2.1	Random Access Response Reception	1194

A.6.2.7.2.2	No Random Access Response Reception	1194

A.6.2.7.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1194

A.6.2.8	E-UTRAN TDD – Non-Contention Based Random Access Test For SCell	1195

A.6.2.8.1	Test Purpose and Environment	1195

A.6.2.8.2	Test Requirements	1197

A.6.2.8.2.1	Random Access Response Reception	1197

A.6.2.8.2.2	No Random Access Response Reception	1197

A.6.2.8.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1197

A.6.2.9	3DL/3UL TDD CA Non-Contention Based Random Access Test for 2 SCells	1198

A.6.2.9.1	Test Purpose and Environment	1198

A.6.2.9.2	Test Requirements	1200

A.6.2.9.2.1	Random Access Response Reception	1200

A.6.2.9.2.2	No Random Access Response Reception	1201

A.6.2.9.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1201

A.6.2.10	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1202

A.6.2.10.1	Test Purpose and Environment	1202

A.6.2.10.2	Test Requirements	1204

A.6.2.10.2.1	Random Access Response Reception	1204

A.6.2.10.2.2	No Random Access Response Reception	1205

A.6.2.10.2.3	Receiving a NACK on msg3	1205

A.6.2.10.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1205

A.6.2.10.2.5	Reception of a Correct Message over Temporary C-RNTI	1205

A.6.2.10.2.6	Contention Resolution Timer expiry	1205

A.6.2.10.2.7	PRACH Resource Selection	1206

A.6.2.11	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1206

A.6.2.11.1	Test Purpose and Environment	1206

A.6.2.11.2	Test Requirements	1208

A.6.2.11.2.1	Random Access Response Reception	1208

A.6.2.11.2.2	No Random Access Response Reception	1209

A.6.2.11.2.3	Receiving a NACK on msg3	1209

A.6.2.11.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1209

A.6.2.11.2.5	Reception of a Correct Message over Temporary C-RNTI	1209

A.6.2.11.2.6	Contention Resolution Timer expiry	1209

A.6.2.11.2.7	PRACH Resource Selection	1210

A.6.2.12	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1210

A.6.2.12.1	Test Purpose and Environment	1210

A.6.2.12.2	Test Requirements	1212

A.6.2.12.2.1	Random Access Response Reception	1212

A.6.2.12.2.2	No Random Access Response Reception	1213

A.6.2.12.2.3	Receiving a NACK on msg3	1213

A.6.2.12.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1213

A.6.2.12.2.5	Reception of a Correct Message over Temporary C-RNTI	1213

A.6.2.12.2.6	Contention Resolution Timer expiry	1213

A.6.2.12.2.7	PRACH Resource Selection	1214

A.6.2.13	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1214

A.6.2.13.1	Test Purpose and Environment	1214

A.6.2.13.2	Test Requirements	1216

A.6.2.13.2.1	Random Access Response Reception	1216

A.6.2.13.2.2	No Random Access Response Reception	1217

A.6.2.13.2.3	Receiving a NACK on msg3	1217

A.6.2.13.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1217

A.6.2.13.2.5	Reception of a Correct Message over Temporary C-RNTI	1217

A.6.2.13.2.6	Contention Resolution Timer expiry	1217

A.6.2.13.2.7	PRACH Resource Selection	1218

A.6.2.14	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1218

A.6.2.14.1	Test Purpose and Environment	1218

A.6.2.14.2	Test Requirements	1220

A.6.2.14.2.1	Random Access Response Reception	1220

A.6.2.14.2.2	No Random Access Response Reception	1221

A.6.2.14.2.3	Receiving a NACK on msg3	1221

A.6.2.14.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1221

A.6.2.14.2.5	Reception of a Correct Message over Temporary C-RNTI	1221

A.6.2.14.2.6	Contention Resolution Timer expiry	1221

A.6.2.14.2.7	PRACH Resource Selection	1222

A.6.2.15	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1222

A.6.2.15.1	Test Purpose and Environment	1222

A.6.2.15.2	Test Requirements	1224

A.6.2.15.2.1	Random Access Response Reception	1224

A.6.2.15.2.2	No Random Access Response Reception	1225

A.6.2.15.2.3	Receiving a NACK on msg3	1225

A.6.2.15.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1225

A.6.2.15.2.5	Reception of a Correct Message over Temporary C-RNTI	1225

A.6.2.15.2.6	Contention Resolution Timer expiry	1225

A.6.2.15.2.7	PRACH Resource Selection	1226

A.6.2.16	Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1226

A.6.2.16.1	Test Purpose and Environment	1226

A.6.2.16.2	Test Requirements	1229

A.6.2.16.2.1	Random Access Response Reception	1229

A.6.2.16.2.2	No Random Access Response Reception	1230

A.6.2.16.2.3	Receiving a NACK on msg3	1230

A.6.2.16.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1230

A.6.2.16.2.5	Reception of a Correct Message over Temporary C-RNTI	1230

A.6.2.16.2.6	Contention Resolution Timer expiry	1230

A.6.2.16.2.7	NPRACH Resource Selection	1230

A.6.2.17	Contention Based Random Access Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1231

A.6.2.17.1	Test Purpose and Environment	1231

A.6.2.17.2	Test Requirements	1234

A.6.2.17.2.1	Random Access Response Reception	1234

A.6.2.17.2.2	No Random Access Response Reception	1235

A.6.2.17.2.3	Receiving a NACK on msg3	1235

A.6.2.17.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1235

A.6.2.17.2.5	Reception of a Correct Message over Temporary C-RNTI	1235

A.6.2.17.2.6	Contention Resolution Timer expiry	1235

A.6.2.17.2.7	NPRACH Resource Selection	1235

A.6.2.18	Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1236

A.6.2.18.1	Test Purpose and Environment	1236

A.6.2.18.2	Test Requirements	1239

A.6.2.18.2.1	Random Access Response Reception	1239

A.6.2.18.2.2	No Random Access Response Reception	1240

A.6.2.18.2.3	Receiving a NACK on msg3	1240

A.6.2.18.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1240

A.6.2.18.2.5	Reception of a Correct Message over Temporary C-RNTI	1240

A.6.2.18.2.6	Contention Resolution Timer expiry	1240

A.6.2.18.2.7	NPRACH Resource Selection	1240

A.6.2.19	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1241

A.6.2.19.1	Test Purpose and Environment	1241

A.6.2.19.2	Test Requirements	1243

A.6.2.19.2.1	Random Access Response Reception	1243

A.6.2.19.2.2	No Random Access Response Reception	1244

A.6.2.19.2.3	Receiving a NACK on msg3	1244

A.6.2.19.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1244

A.6.2.19.2.5	Reception of a Correct Message over Temporary C-RNTI	1244

A.6.2.19.2.6	Contention Resolution Timer expiry	1244

A.6.2.19.2.7	NPRACH Resource Selection	1244

A.6.2.20	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in enhanced coverage	1245

A.6.2.20.1	Test Purpose and Environment	1245

A.6.2.20.2	Test Requirements	1247

A.6.2.20.2.1	Random Access Response Reception	1247

A.6.2.20.2.2	No Random Access Response Reception	1248

A.6.2.20.2.3	Receiving a NACK on msg3	1248

A.6.2.20.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1248

A.6.2.20.2.5	Reception of a Correct Message over Temporary C-RNTI	1248

A.6.2.20.2.6	Contention Resolution Timer expiry	1248

A.6.2.20.2.7	NPRACH Resource Selection	1248

A.6.2.21	TDD Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1249

A.6.2.21.1	Test Purpose and Environment	1249

A.6.2.21.2	Test Requirements	1251

A.6.2.21.2.1	Random Access Response Reception	1251

A.6.2.21.2.2	No Random Access Response Reception	1252

A.6.2.21.2.3	Receiving a NACK on msg3	1252

A.6.2.21.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1252

A.6.2.21.2.5	Reception of a Correct Message over Temporary C-RNTI	1252

A.6.2.21.2.6	Contention Resolution Timer expiry	1252

A.6.2.21.2.7	NPRACH Resource Selection	1252

A.6.3	RRC Connection Release with Redirection	1253

A.6.3.1	Redirection from E-UTRAN FDD to UTRAN FDD	1253

A.6.3.1.1	Test Purpose and Environment	1253

A.6.3.1.2	Test Requirements	1255

A.6.3.2	Redirection from E-UTRAN TDD to UTRAN FDD	1255

A.6.3.2.1	Test Purpose and Environment	1255

A.6.3.2.2	Test Requirements	1257

A.6.3.3	Redirection from E-UTRAN FDD to GERAN when System Information is provided	1257

A.6.3.3.1	Test Purpose and Environment	1257

A.6.3.3.2	Test Requirements	1258

A.6.3.4	Redirection from E-UTRAN TDD to GERAN when System Information is provided	1259

A.6.3.4.1	Test Purpose and Environment	1259

A.6.3.4.2	Test Requirements	1260

A.6.3.5	E-UTRA TDD RRC connection release redirection to UTRA TDD	1261

A.6.3.5.1	Test Purpose and Environment	1261

A.6.3.5.2	Test Requirements	1263

A.6.3.6	E-UTRA FDD RRC connection release redirection to UTRA TDD	1263

A.6.3.6.1	Test Purpose and Environment	1263

A. 6.3.6.2	Test Requirements	1265

A.6.3.7	E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided	1265

A.6.3.7.1	Test Purpose and Environment	1265

A.6.3.7.2	Test Requirements	1268

A.6.3.8	E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided	1268

A.6.3.8.1	Test Purpose and Environment	1268

A.6.3.8.2	Test Requirements	1271

A.6.3.9	Redirection from E-UTRAN FDD to UTRAN FDD without System Information	1271

A.6.3.9.1	Test Purpose and Environment	1271

A.6.3.9.2	Test Requirements	1273

A.6.3.10	Redirection from E-UTRAN FDD to GERAN when System Information is not provided	1273

A.6.3.10.1	Test Purpose and Environment	1273

A.6.3.10.2	Test Requirements	1275

A.6.3.11	Redirection from E-UTRAN TDD to GERAN when System Information is not provided	1275

A.6.3.11.1	Test Purpose and Environment	1275

A.6.3.11.2	Test Requirements	1277

A.6.3.12	E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided	1277

A.6.3.12.1	Test Purpose and Environment	1277

A.6.3.12.2	Test Requirements	1280

A.7	Timing and Signalling Characteristics	1281

A.7.1	UE Transmit Timing	1281

A.7.1.1	E-UTRAN FDD – UE Transmit Timing Accuracy Tests	1281

A.7.1.1.1	Test Purpose and Environment	1281

A.7.1.1.2	Test Requirements	1282

A.7.1.2	E-UTRAN TDD - UE Transmit Timing Accuracy Tests	1283

A.7.1.2.1	Test Purpose and Environment	1283

A.7.1.2.2	Test Requirements	1285

A.7.1.3	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell	1286

A.7.1.3.1	Test Purpose and Environment	1286

A.7.1.3.2	Test Requirements	1288

A.7.1.4	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell	1289

A.7.1.4.1	Test Purpose and Environment	1289

A.7.1.4.2	Test Requirements	1290

A.7.1.4A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell for 20 MHz + 10 MHz	1291

A.7.1.4A.1	Test Purpose and Environment	1291

A.7.1.4A.2	Test Requirements	1291

A.7.1.5	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for 5MHz Bandwidth	1291

A.7.1.5.1	Test Purpose and Environment	1291

A.7.1.5.2	Test Requirements	1292

A.7.1.6	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell in sTAG	1292

A.7.1.6.1	Test Purpose and Environment	1292

A.7.1.6.2	Test Requirements	1294

A.7.1.7	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG	1295

A.7.1.7.1	Test Purpose and Environment	1295

A.7.1.7.2	Test Requirements	1296

A.7.1.7A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +20MHz	1297

A.7.1.7A.1	Test Purpose and Environment	1297

A.7.1.7A.2	Test Requirements	1297

A.7.1.7B	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +10MHz	1297

A.7.1.7B.1	Test Purpose and Environment	1297

A.7.1.7B.2	Test Requirements	1297

A.7.1.8	Void	1298

A.7.1.8.1	Void	1298

A.7.1.8.2	Void	1298

A.7.1.9	Void	1298

A.7.1.9.1	Void	1298

A.7.1.9.2	Void	1298

A.7.1.10	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1298

A.7.1.10.1	Test Purpose and Environment	1298

A.7.1.10.2	Test Requirements	1300

A.7.1.11	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1300

A.7.1.11.1	Test Purpose and Environment	1300

A.7.1.11.2	Test Requirements	1302

A.7.1.12	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1302

A.7.1.12.1	Test Purpose and Environment	1302

A.7.1.12.2	Test Requirements	1304

A.7.1.13	3DL/3UL TDD CA UE Transmit Timing Accuracy Tests for 2 SCells	1304

A.7.1.13.1	Test Purpose and Environment	1304

A.7.1.13.2	Test Requirements	1306

A.7.1.14	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1307

A.7.1.14.1	Test Purpose and Environment	1307

A.7.1.14.2	Test Requirements	1308

A.7.1.15	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1308

A.7.1.15.1	Test Purpose and Environment	1308

A.7.1.15.2	Test Requirements	1309

A.7.1.16	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1310

A.7.1.16.1	Test Purpose and Environment	1310

A.7.1.16.2	Test Requirements	1311

A.7.1.17	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1312

A.7.1.17.1	Test Purpose and Environment	1312

A.7.1.17.2	Test Requirements	1314

A.7.1.18	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1315

A.7.1.18.1	Test Purpose and Environment	1315

A.7.1.18.2	Test Requirements	1317

A.7.1.19	E-UTRAN FDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1318

A.7.1.19.1	Test Purpose and Environment	1318

A.7.1.19.2	Test Requirements	1319

A.7.1.20	E-UTRAN TDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1319

A.7.1.20.1	Test Purpose and Environment	1319

A.7.1.20.2	Test Requirements	1321

A.7.1.21	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1321

A.7.1.21.1	Test Purpose and Environment	1321

A.7.1.21.2	Test Requirements	1323

A.7.1.22	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1323

A.7.1.22.1	Test Purpose and Environment	1323

A.7.1.22.2	Test Requirements	1325

A.7.1.23	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1325

A.7.1.23.1	Test Purpose and Environment	1325

A.7.1.23.2	Test Requirements	1327

A.7.1.24	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1327

A.7.1.24.1	Test Purpose and Environment	1327

A.7.1.24.2	Test Requirements	1328

A.7.1.25	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1329

A.7.1.25.1	Test Purpose and Environment	1329

A.7.1.25.2	Test Requirements	1330

A.7.1.26	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1331

A.7.1.26.1	Test Purpose and Environment	1331

A.7.1.26.2	Test Requirements	1332

A.7.1.27	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1333

A.7.1.27.1	Test Purpose and Environment	1333

A.7.1.27.2	Test Requirements	1335

A.7.1.28	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1335

A.7.1.28.1	Test Purpose and Environment	1335

A.7.1.28.2	Test Requirements	1337

A.7.2	UE Timing Advance	1338

A.7.2.1	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test	1338

A.7.2.1.1	Test Purpose and Environment	1338

A.7.2.1.2	Test Requirements	1339

A.7.2.2	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test	1340

A.7.2.2.1	Test Purpose and Environment	1340

A.7.2.2.2	Test Requirements	1342

A.7.2.3	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for 5MHz	1342

A.7.2.3.1	Test Purpose and Environment	1342

A.7.2.3.2	Test Requirements	1342

A.7.2.4	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for SCell in sTAG	1342

A.7.2.4.1	Test Purpose and Environment	1342

A.7.2.4.2	Test Requirements	1345

A.7.2.5	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG	1345

A.7.2.5.1	Test Purpose and Environment	1345

A.7.2.5.2	Test Requirements	1347

A.7.2.5A	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +20 MHz	1347

A.7.2.5A.1	Test Purpose and Environment	1347

A.7.2.5A.2	Test Requirements	1348

A.7.2.5B	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +10 MHz	1348

A.7.2.5B.1	Test Purpose and Environment	1348

A.7.2.5B.2	Test Requirements	1348

A.7.2.6	E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1348

A.7.2.6.1	Test Purpose and Environment	1348

A.7.2.6.2	Test Requirements	1351

A.7.2.7	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1351

A.7.2.7.1	Test Purpose and Environment	1351

A.7.2.7.2	Test Requirements	1353

A.7.2.8	E-UTRAN TDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1353

A.7.2.8.1	Test Purpose and Environment	1353

A.7.2.8.2	Test Requirements	1355

A.7.2.9.2	Test Requirements	1356

A.7.2.10	E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1357

A.7.2.10.1	Test Purpose and Environment	1357

A.7.2.10.2	Test Requirements	1358

A.7.2.11	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1358

A.7.2.11.1	Test Purpose and Environment	1358

A.7.2.11.2	Test Requirements	1360

A.7.2.12	E-UTRAN TDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1360

A.7.2.12.1	Test Purpose and Environment	1360

A.7.2.12.2	Test Requirements	1362

A.7.2.13	E-UTRAN FDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1362

A.7.2.13.1	Test Purpose and Environment	1362

A.7.2.13.2	Test Requirements	1364

A.7.2.14	E-UTRAN TDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1364

A.7.2.14.1	Test Purpose and Environment	1364

A.7.2.14.2	Test Requirements	1366

A.7.2.15	E-UTRAN TDD – TDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhanced Coverage	1366

A.7.2.15.1	Test Purpose and Environment	1366

A.7.2.15.2	Test Requirements	1368

A.7.3	Radio Link Monitoring	1368

A.7.3.1	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync	1369

A.7.3.1.1	Test Purpose and Environment	1369

A.7.3.1.2	Test Requirements	1373

A.7.3.2	E-UTRAN FDD Radio Link Monitoring Test for In-sync	1373

A.7.3.2.1	Test Purpose and Environment	1373

A.7.3.2.2	Test Requirements	1376

A.7.3.3	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync	1376

A.7.3.3.1	Test Purpose and Environment	1376

A.7.3.3.2	Test Requirements	1380

A.7.3.4	E-UTRAN TDD Radio Link Monitoring Test for In-sync	1380

A.7.3.4.1	Test Purpose and Environment	1380

A.7.3.4.2	Test Requirements	1383

A.7.3.5	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX	1383

A.7.3.5.1	Test Purpose and Environment	1383

A.7.3.5.2	Test Requirements	1387

A.7.3.6	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX	1388

A.7.3.6.1	Test Purpose and Environment	1388

A.7.3.6.2	Test Requirements	1391

A.7.3.7	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX	1391

A.7.3.7.1	Test Purpose and Environment	1391

A.7.3.7.2	Test Requirements	1394

A.7.3.8	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX	1394

A.7.3.8.1	Test Purpose and Environment	1394

A.7.3.8.2	Test Requirements	1397

A.7.3.9 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction and Non-MBSFN ABS	1397

A.7.3.9.2	Test Requirements	1400

A.7.3.10	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1400

A.7.3.10.1	Test Purpose and Environment	1400

A.7.3.11	E-UTRAN FDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1403

A.7.3.11.1	Test Purpose and Environment	1403

A.7.3.11.2	Test Requirements	1407

A.7.3.12	E-UTRAN TDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1407

A.7.3.12.1	Test Purpose and Environment	1407

A.7.3.12.2	Test Requirements	1411

A.7.3.13 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1411

A.7.3.13.1	Test Purpose and Environment	1411

A.7.3.13.2	Test Requirements	1414

A.7.3.14 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1414

A.7.3.14.1	Test Purpose and Environment	1414

A.7.3.14.2	Test Requirements	1417

A.7.3.15	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1417

A.7.3.15.1	Test Purpose and Environment	1417

A.7.3.15.2	Test Requirements	1420

A.7.3.16	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1420

A.7.3.16.1	Test Purpose and Environment	1420

A.7.3.16.2	Test Requirements	1424

A.7.3.17	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1424

A.7.3.17.1	Test Purpose and Environment	1424

A.7.3.17.2	Test Requirements	1427

A.7.3.18	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1427

A.7.3.18.1	Test Purpose and Environment	1427

A.7.3.18.2	Test Requirements	1431

A.7.3.19	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1431

A.7.3.19.1	Test Purpose and Environment	1431

A.7.3.19.2	Test Requirements	1435

A.7.3.20	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1435

A.7.3.20.1	Test Purpose and Environment	1435

A.7.3.20.2	Test Requirements	1439

A.7.3.21	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1439

A.7.3.21.1	Test Purpose and Environment	1439

A.7.3.21.2	Test Requirements	1443

A.7.3.22	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1443

A.7.3.22.1	Test Purpose and Environment	1443

A.7.3.22.2	Test Requirements	1447

A.7.3.23	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth	1447

A.7.3.23.1	Test Purpose and Environment	1447

A.7.3.23.2	Test Requirements	1448

A.7.3.24	E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth	1448

A.7.3.24.1	Test Purpose and Environment	1448

A.7.3.24.2	Test Requirements	1449

A.7.3.25	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth	1449

A.7.3.25.1	Test Purpose and Environment	1449

A.7.3.25.2	Test Requirements	1450

A.7.3.26	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1450

A.7.3.26.1	Test Purpose and Environment	1450

A.7.3.26.2	Test Requirements	1453

A.7.3.27	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1453

A.7.3.27.1	Test Purpose and Environment	1453

A.7.3.27.2	Test Requirements	1456

A.7.3.28	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1456

A.7.3.28.1	Test Purpose and Environment	1456

A.7.3.28.2	Test Requirements	1459

A.7.3.29	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1459

A.7.3.29.1	Test Purpose and Environment	1459

A.7.3.29.2	Test Requirements	1462

A.7.3.30	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1462

A.7.3.30.1	Test Purpose and Environment	1462

A.7.3.30.2	Test Requirements	1465

A.7.3.31	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1465

A.7.3.31.1	Test Purpose and Environment	1465

A.7.3.31.2	Test Requirements	1468

A.7.3.32	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1468

A.7.3.32.1	Test Purpose and Environment	1468

A.7.3.32.2	Test Requirements	1471

A.7.3.33	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1471

A.7.3.33.1	Test Purpose and Environment	1471

A.7.3.33.2	Test Requirements	1474

A.7.3.34	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1474

A.7.3.34.1	Test Purpose and Environment	1474

A.7.3.34.2	Test Requirements	1477

A.7.3.35	E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE category 0	1477

A.7.3.35.1	Test Purpose and Environment	1477

A.7.3.35.2	Test Requirements	1480

A.7.3.36	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1480

A.7.3.36.1	Test Purpose and Environment	1480

A.7.3.36.2	Test Requirements	1483

A.7.3.37	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE category 0	1483

A.7.3.37.1	Test Purpose and Environment	1483

A.7.3.37.2	Test Requirements	1486

A.7.3.38	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1486

A.7.3.38.1	Test Purpose and Environment	1486

A.7.3.38.2	Test Requirements	1489

A.7.3.39	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC	1489

A.7.3.39.1	Test Purpose and Environment	1489

A.7.3.39.2	Test Requirements	1492

A.7.3.40	E-UTRAN TDD-TDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1493

A.7.3.40.1	Test Purpose and Environment	1493

A.7.3.40.2	Test Requirements	1495

A.7.3.41	E-UTRAN FDD-FDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1496

A.7.3.41.1	Test Purpose and Environment	1496

A.7.3.41.2	Test Requirements	1499

A.7.3.42	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for In-sync in DRX in asynchronous DC	1499

A.7.3.42.1	Test Purpose and Environment	1499

A.7.3.42.2	Test Requirements	1502

A.7.3.43	E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1502

A.7.3.43.1	Test Purpose and Environment	1502

A.7.3.43.2	Test Requirements	1505

A.7.3.44	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in FDD	1505

A.7.3.44.1	Test Purpose and Environment	1505

A.7.3.44.2	Test Requirements	1508

A.7.3.45	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in TDD	1509

A.7.3.45.1	Test Purpose and Environment	1509

A.7.3.45.2	Test Requirements	1511

A.7.3.46	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in FDD	1512

A.7.3.46.1	Test Purpose and Environment	1512

A.7.3.46.2	Test Requirements	1515

A.7.3.47	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in TDD	1515

A.7.3.47.1	Test Purpose and Environment	1515

A.7.3.47.2	Test Requirements	1518

A.7.3.48	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1518

A.7.3.48.1	Test Purpose and Environment	1518

A.7.3.48.2	Test Requirements	1521

A.7.3.49	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1521

A.7.3.49.1	Test Purpose and Environment	1521

A.7.3.49.2	Test Requirements	1524

A.7.3.50	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1524

A.7.3.50.1	Test Purpose and Environment	1524

A.7.3.50.2	Test Requirements	1527

A.7.3.51	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1527

A.7.3.51.1	Test Purpose and Environment	1527

A.7.3.51.2	Test Requirements	1530

A.7.3.52	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1530

A.7.3.52.1	Test Purpose and Environment	1530

A.7.3.52.2	Test Requirements	1533

A.7.3.53	E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1533

A.7.3.53.1	Test Purpose and Environment	1533

A.7.3.53.2	Test Requirements	1536

A.7.3.54	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1536

A.7.3.54.1	Test Purpose and Environment	1536

A.7.3.54.2	Test Requirements	1539

A.7.3.55	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1539

A.7.3.55.1	Test Purpose and Environment	1539

A.7.3.55.2	Test Requirements	1542

A.7.3.56	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1542

A.7.3.56.1	Test Purpose and Environment	1542

A.7.3.56.2	Test Requirements	1545

A.7.3.57	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1545

A.7.3.57.1	Test Purpose and Environment	1545

A.7.3.57.2	Test Requirements	1548

A.7.3.58	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1548

A.7.3.58.1	Test Purpose and Environment	1548

A.7.3.58.2	Test Requirements	1551

A.7.3.59	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1551

A.7.3.59.1	Test Purpose and Environment	1551

A.7.3.59.2	Test Requirements	1554

A.7.3.60	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1554

A.7.3.60.1	Test Purpose and Environment	1554

A.7.3.60.2	Test Requirements	1558

A.7.3.61	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1558

A.7.3.61.1	Test Purpose and Environment	1558

A.7.3.61.2	Test Requirements	1562

A.7.3.62	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1562

A.7.3.62.1	Test Purpose and Environment	1562

A.7.3.62.2	Test Requirements	1566

A.7.3.63	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1566

A.7.3.63.1	Test Purpose and Environment	1566

A.7.3.63.2	Test Requirements	1570

A.7.3.64	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1570

A.7.3.64.1	Test Purpose and Environment	1570

A.7.3.64.2	Test Requirements	1573

A.7.3.65	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1574

A.7.3.65.1	Test Purpose and Environment	1574

A.7.3.65.2	Test Requirements	1577

A.7.3.66	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1578

A.7.3.66.1	Test Purpose and Environment	1578

A.7.3.66.2	Test Requirements	1580

A.7.3.67	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1581

A.7.3.67.1	Test Purpose and Environment	1581

A.7.3.67.2	Test Requirements	1584

A.7.3.68	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1585

A.7.3.68.1	Test Purpose and Environment	1585

A.7.3.68.2	Test Requirements	1587

A.7.3.69	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1587

A.7.3.69.1	Test Purpose and Environment	1587

A.7.3.69.2	Test Requirements	1589

A.7.3.70	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1589

A.7.3.70.1	Test Purpose and Environment	1589

A.7.3.70.2	Test Requirements	1591

A.7.3.71	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1591

A.7.3.71.1	Test Purpose and Environment	1591

A.7.3.71.2	Test Requirements	1594

A.7.3.72	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1594

A.7.3.72.1	Test Purpose and Environment	1594

A.7.3.72.2	Test Requirements	1596

A.7.3.73	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1596

A.7.3.73.1	Test Purpose and Environment	1596

A.7.3.73.2	Test Requirements	1598

A.7.3.74	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1598

A.7.3.74.1	Test Purpose and Environment	1598

A.7.3.74.2	Test Requirements	1601

A.7.3.75	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1601

A.7.3.75.1	Test Purpose and Environment	1601

A.7.3.75.2	Test Requirements	1603

A.7.3.76	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1604

A.7.3.76.1	Test Purpose and Environment	1604

A.7.3.76.2	Test Requirements	1607

A.7.3.77	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1607

A.7.3.77.1	Test Purpose and Environment	1607

A.7.3.77.2	Test Requirements	1610

A.7.3.78	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1610

A.7.3.78.1	Test Purpose and Environment	1610

A.7.3.78.2	Test Requirements	1613

A.7.3.79	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1613

A.7.3.79.1	Test Purpose and Environment	1613

A.7.3.79.2	Test Requirements	1616

A.7.3.80	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1616

A.7.3.80.1	Test Purpose and Environment	1616

A.7.3.80.2	Test Requirements	1619

A.7.3.81	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1619

A.7.3.81.1	Test Purpose and Environment	1619

A.7.3.81.2	Test Requirements	1622

A.7.3.82	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1622

A.7.3.82.1	Test Purpose and Environment	1622

A.7.3.82.2	Test Requirements	1625

A.7.3.83	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1625

A.7.3.83.1	Test Purpose and Environment	1625

A.7.3.83.2	Test Requirements	1626

A.7.3.84	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1627

A.7.3.84.1	Test Purpose and Environment	1627

A.7.3.84.2	Test Requirements	1629

A.7.3.85	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1629

A.7.3.85.1	Test Purpose and Environment	1629

A.7.3.85.2	Test Requirements	1630

A.7.3.86	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1631

A.7.3.86.1	Test Purpose and Environment	1631

A.7.3.86.2	Test Requirements	1633

A.7.3.87	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1633

A.7.3.87.1	Test Purpose and Environment	1633

A.7.3.87.2	Test Requirements	1635

A.7.3.88	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1636

A.7.3.88.1	Test Purpose and Environment	1636

A.7.3.88.2	Test Requirements	1640

A.7.3.89	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1640

A.7.3.89.1	Test Purpose and Environment	1640

A.7.3.89.2	Test Requirements	1644

A.7.3.90	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1644

A.7.3.90.1	Test Purpose and Environment	1644

A.7.3.90.2	Test Requirements	1648

A.7.3.91	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1648

A.7.3.91.1	Test Purpose and Environment	1648

A.7.3.91.2	Test Requirements	1652

A.7.3.92	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1652

A.7.3.92.1	Test Purpose and Environment	1652

A.7.3.92.2	Test Requirements	1655

A.7.3.93	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1656

A.7.3.93.1	Test Purpose and Environment	1656

A.7.3.93.2	Test Requirements	1659

A.7.3.94	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1660

A.7.3.94.1	Test Purpose and Environment	1660

A.7.3.94.2	Test Requirements	1662

A.7.3.95	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1663

A.7.3.95.1	Test Purpose and Environment	1663

A.7.3.95.2	Test Requirements	1666

A.7.4	Interruption for Dual Connectivity	1667

A.7.4.1	E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1667

A.7.4.1.1	Test Purpose and Environment	1667

A.7.4.1.2	Test Requirements	1669

A.7.4.2	E-UTRAN TDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1669

A.7.4.2.1	Test Purpose and Environment	1669

A.7.4.2.2	Test Requirements	1671

A.7.4.3	E-UTRAN FDD-FDD Interruption at transitions between active and non-active during DRX in asynchronous dual connectivity	1671

A.7.4.3.1	Test Purpose and Environment	1671

A.7.4.3.2	Test Requirements	1673

A.7.4.4	E-UTRAN FDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1673

A.7.4.4.1	Test Purpose and Environment	1673

A.7.4.4.2	Test Requirements	1676

A.7.4.5	E-UTRAN TDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1676

A.7.4.5.1	Test Purpose and Environment	1676

A.7.4.5.2	Test Requirements	1678

A.7.4.6	E-UTRAN FDD-TDD DC interruption at SRS carrier based switching	1678

A.7.4.6.1	Test Purpose and Environment	1678

A.7.4.6.2	Test Requirements	1680

A.7.4.7	E-UTRAN TDD-TDD DC interruption at SRS carrier based switching	1680

A.7.4.7.1	Test Purpose and Environment	1680

A.7.4.7.2	Test Requirements	1683

A.7.5	Proximity-based Services	1683

A.7.5.1	E-UTRAN FDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1683

A.7.5.1.1	Test Purpose and Environment	1683

A.7.5.1.2	Test Requirements	1684

A.7.5.2	E-UTRAN TDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1685

A.7.5.2.1	Test Purpose and Environment	1685

A.7.5.1.2	Test Requirements	1685

A.7.5.3	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery	1686

A.7.5.3.1	Test Purpose and Environment	1686

A.7.5.3.2	Test Requirements	1687

A.7.5.4	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	1688

A.7.5.4.1	Test Purpose and Environment	1688

A.7.5.4.2	Test Requirements	1689

A.7.5.5	E-UTRAN FDD - Interruptions due to ProSe Direct Communication	1690

A.7.5.5.1	Test Purpose and Environment	1690

A.7.5.5.2	Test Requirements	1692

A.7.5.6	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery with discovery period less than 320ms	1693

A.7.5.6.1	Test Purpose and Environment	1693

A.7.5.6.2	Test Requirements	1694

A.7.5.7	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery	1695

A.7.5.7.1	Test Purpose and Environment	1695

A.7.5.7.2	Test Requirements	1697

A.7.5.8	E-UTRAN FDD-FDD - Cell reselection and timing accuracy for ProSe Direct Discovery transmission on non-serving frequency	1697

A.7.5.8.1	Test Purpose and Environment	1697

A.7.5.8.2	Test Requirements	1699

A.7.5.9	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery reception on non-serving frequency	1700

A.7.5.9.1	Test Purpose and Environment	1700

A.7.5.9.2	Test Requirements	1702

A.7.5.10	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery transmission on non-serving frequency	1703

A.7.5.10.1	Test Purpose and Environment	1703

A.7.5.10.2	Test Requirements	1705

A.7.5.11	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Communication on non-serving frequency	1706

A.7.5.11.1	Test Purpose and Environment	1706

A.7.5.11.2	Test Requirements	1708

A.7.5.12	E-UTRAN FDD - Selection / Reselection of ProSe relay UE	1708

A.7.5.12.1	Test Purpose and Environment	1708

A.7.5.12.2	Test Requirements	1712

A.7.6	Interruption for carrier aggregation	1713

A.7.6.1	E-UTRAN FDD-TDD CA interruption at SRS carrier based switching	1713

A.7.6.1.1	Test Purpose and Environment	1713

A.7.6.1.2	Test Requirements	1715

A.7.6.2	E-UTRAN TDD-TDD CA interruption at SRS carrier based switching	1715

A.7.6.2.1	Test Purpose and Environment	1715

A.7.6.2.2	Test Requirements	1718

A.8	UE Measurements Procedures	1718

A.8.1	E-UTRAN FDD Intra-frequency Measurements	1718

A.8.1.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1718

A.8.1.1.1	Test Purpose and Environment	1718

A.8.1.1.2	Test Requirements	1720

A.8.1.2	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1720

A.8.1.2.1	Test Purpose and Environment	1720

A.8.1.2.2	Test Requirements	1722

A.8.1.3	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1722

A.8.1.3.1	Test Purpose and Environment	1722

A.8.1.3.2	Test Requirements	1725

A.8.1.4	Void	1725

A.8.1.5	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1725

A.8.1.5.1	Test Purpose and Environment	1725

A.8.1.5.2	Test Requirements	1727

A.8.1.6	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1727

A.8.1.6.1	Test Purpose and Environment	1727

A.8.1.6.2	Test Requirements	1729

A.8.1.7 	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1729

A.8.1.7.1	Test Purpose and Environment	1729

A.8.1.7.2	Test Requirements	1731

A.8.1.8	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1732

A.8.1.8.1	Test Purpose and Environment	1732

A.8.1.8.2	Test Requirements	1734

A.8.1.9	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for 5MHz bandwidth	1735

A.8.1.9.1	Test Purpose and Environment	1735

A.8.1.9.2	Test Requirements	1735

A.8.1.10	E-UTRAN FDD-FDD Intra-Frequency Event Triggered Reporting under Fading Propagation Conditions in Synchronous Cells with DRX for 5 MHz Bandwidth	1735

A.8.1.10.1	Test Purpose and Environment	1735

A.8.1.10.2	Test Requirements	1736

A.8.1.11	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1736

A.8.1.11.1	Test Purpose and Environment	1736

A.8.1.11.2	Test Requirements	1738

A.8.1.12	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1739

A.8.1.12.1	Test Purpose and Environment	1739

A.8.1.12.2	Test Requirements	1740

A.8.1.13	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1741

A.8.1.13.1	Test Purpose and Environment	1741

A.8.1.13.2	Test Requirements	1743

A.8.1.14	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1743

A.8.1.14.1	Test Purpose and Environment	1743

A.8.1.14.2	Test Requirements	1745

A.8.1.15	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1746

A.8.1.15.1	Test Purpose and Environment	1746

A.8.1.15.2	Test Requirements	1747

A.8.1.16	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1748

A.8.1.16.1	Test Purpose and Environment	1748

A.8.1.16.2	Test Requirements	1750

A.8.1.17	Void	1750

A.8.1.18	Void	1750

A.8.1.19	E-UTRAN FDD-FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1750

A.8.1.19.1	Test Purpose and Environment	1750

A.8.1.19.2	Test Requirements	1752

A.8.1.20	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1753

A.8.1.20.1	Test Purpose and Environment	1753

A.8.1.20.2	Test Requirements	1755

A.8.1.21	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1755

A.8.1.21.1	Test Purpose and Environment	1755

A.8.1.21.2	Test Requirements	1757

A.8.1.22	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1758

A.8.1.22.1	Test Purpose and Environment	1758

A.8.1.22.2	Test Requirements	1760

A.8.1.23	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1760

A.8.1.23.1	Test Purpose and Environment	1760

A.8.1.23.2	Test Requirements	1761

A.8.1.24	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1762

A.8.1.24.1	Test Purpose and Environment	1762

A.8.1.24.2	Test Requirements	1764

A.8.1.25	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1764

A.8.1.25.1	Test Purpose and Environment	1764

A.8.1.25.2	Test Requirements	1766

A.8.1.26	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1766

A.8.1.26.1	Test Purpose and Environment	1766

A.8.1.27	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1769

A.8.1.27.1	Test Purpose and Environment	1769

A.8.1.27.2	Test Requirements	1771

A.8.1.28	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1771

A.8.1.28.1	Test Purpose and Environment	1771

A.8.1.28.2	Test Requirements	1773

A.8.1.29	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1773

A.8.1.29.1	Test Purpose and Environment	1773

A.8.1.29.2	Test Requirements	1775

A.8.1.30	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1776

A.8.1.30.1	Test Purpose and Environment	1776

A.8.1.30.2	Test Requirements	1778

A.8.1.31 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1778

A.8.1.31.1	Test Purpose and Environment	1778

A.8.1.31.2	Test Requirements	1780

A.8.1.32 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1781

A.8.1.32.1	Test Purpose and Environment	1781

A.8.1.32.2	Test Requirements	1782

A.8.1.33	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1783

A.8.1.33.1	Test Purpose and Environment	1783

A.8.1.33.2	Test Requirements	1784

A.8.1.34	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1785

A.8.1.34.1	Test Purpose and Environment	1785

A.8.1.34.2	Test Requirements	1786

A.8.1.35	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1787

A.8.1.35.1	Test Purpose and Environment	1787

A.8.1.35.2	Test Requirements	1788

A.8.1.36	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1789

A.8.1.36.1	Test Purpose and Environment	1789

A.8.1.36.2	Test Requirements	1790

A.8.1.37	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1791

A.8.1.37.1	Test Purpose and Environment	1791

A.8.1.37.2	Test Requirements	1793

A.8.1.38	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1793

A.8.1.38.1	Test Purpose and Environment	1793

A.8.1.38.2	Test Requirements	1794

A.8.1.39	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1795

A.8.1.39.1	Test Purpose and Environment	1795

A.8.1.39.2	Test Requirements	1797

A.8.1.40	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	1797

A.8.1.40.1	Test Purpose and Environment	1797

A.8.1.40.2	Test Requirements	1799

A.8.1.41	E-UTRAN FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1799

A.8.1.41.1	Test Purpose and Environment	1799

A.8.1.41.2	Test Requirement	1801

A.8.1.42	E-UTRAN HD-FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1801

A.8.1.42.1	Test Purpose and Environment	1801

A.8.1.42.2	Test Requirement	1802

A.8.2	E-UTRAN TDD Intra-frequency Measurements	1803

A.8.2.1	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1803

A.8.2.1.1	Test Purpose and Environment	1803

A.8.2.1.2	Test Requirements	1804

A.8.2.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1804

A.8.2.2.1	Test Purpose and Environment	1804

A.8.2.2.2	Test Requirements	1807

A.8.2.3	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1807

A.8.2.3.1	Test Purpose and Environment	1807

A.8.2.3.2	Test Requirements	1809

A.8.2.4	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1810

A.8.2.4.1	Test Purpose and Environment	1810

A.8.2.4.2	Test Requirements	1812

A.8.2.5	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1812

A.8.2.5.1	Test Purpose and Environment	1812

A.8.2.5.2	Test Requirements	1814

A.8.2.6	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1815

A.8.2.6.1	Test Purpose and Environment	1815

A.8.2.6.2	Test Requirements	1817

A.8.2.7	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1818

A.8.2.7.1	Test Purpose and Environment	1818

A.8.2.7.2	Test Requirements	1819

A.8.2.8	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1820

A.8.2.8.1	Test Purpose and Environment	1820

A.8.2.8.2	Test Requirements	1822

A.8.2.9	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1822

A.8.2.9.1	Test Purpose and Environment	1822

A.8.2.9.2	Test Requirements	1824

A.8.2.10	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1825

A.8.2.10.1	Test Purpose and Environment	1825

A.8.2.10.2	Test Requirements	1827

A.8.2.11	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	1827

A.8.2.11.1	Test Purpose and Environment	1827

A.8.2.11.2	Test Requirements	1829

A.8.2.12	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1829

A.8.2.12.1	Test Purpose and Environment	1829

A.8.2.12.2	Test Requirements	1831

A.8.2.13	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1832

A.8.2.13.1	Test Purpose and Environment	1832

A.8.2.13.2	Test Requirements	1834

A.8.2.14	E-UTRAN TDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1834

A.8.2.14.1	Test Purpose and Environment	1834

A.8.2.14.2	Test Requirement	1836

A.8.3	E-UTRAN FDD - FDD Inter-frequency Measurements	1837

A.8.3.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1837

A.8.3.1.1	Test Purpose and Environment	1837

A.8.3.1.2	Test Requirements	1838

A.8.3.2	E-UTRAN FDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	1838

A.8.3.2.1	Test Purpose and Environment	1838

A.8.3.2.2	Test Requirements	1841

A.8.3.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used	1841

A.8.3.3.1	Test Purpose and Environment	1841

A.8.3.3.2	Test Requirements	1844

A.8.3.4	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1844

A.8.3.4.1	Test Purpose and Environment	1844

A.8.3.4.2	Test Requirements	1846

A.8.3.5	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1846

A.8.3.5.2	Test Requirements	1848

A.8.3.6	E-UTRAN FDD-FDD Inter-frequency event triggered reporting without measurement gaps under AWGN propagation conditions in asynchronous cells	1848

A.8.3.6.1	Test Purpose and Environment	1848

A.8.3.6.2	Test Requirements	1850

A.8.3.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group	1851

A.8.3.7.1	Test Purpose and Environment	1851

A.8.3.7.2	Test Requirements	1853

A.8.3.8	FDD-FDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	1854

A.8.3.8.1	Test Purpose and Environment	1854

A.8.3.8.2	Test Requirements	1857

A.8.3.9	FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	1857

A.8.3.9.1	Test Purpose and Environment	1857

A.8.3.9.2	Test Requirements	1860

A.8.3.10	E-UTRAN FDD-FDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	1861

A.8.3.10.1	Test Purpose and Environment	1861

A.8.3.10.2	Test Requirements	1862

A.8.3.11	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells with burst gap	1862

A.8.3.11.1	Test Purpose and Environment	1862

A.8.3.11.2	Test Requirement	1864

A.8.3.12	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	1865

A.8.3.12.1	Test Purpose and Environment	1865

A.8.3.12.2	Test Requirement	1866

A.8.3.13	E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	1867

A.8.3.13.1	Test Purpose and Environment	1867

A.8.3.13.2	Test Requirement	1868

A.8.3.14	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	1869

A.8.3.14.1	Test Purpose and Environment	1869

A.8.3.14.2	Test Requirement	1870

A.8.3.15	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	1871

A.8.3.15.1	Test Purpose and Environment	1871

A.8.3.15.2	Test Requirement	1872

A.8.3.16	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used	1873

A.8.3.16.1	Test Purpose and Environment	1873

A.8.3.16.2	Test Requirement	1875

A.8.3.17	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	1875

A.8.3.17.1	Test Purpose and Environment	1875

A.8.3.17.2	Test Requirement	1878

A.8.3.18	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	1878

A.8.3.18.1	Test Purpose and Environment	1878

A.8.3.18.2	Test Requirement	1881

A.8.3.19	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	1881

A.8.3.19.1	Test Purpose and Environment	1881

A.8.3.19.2	Test Requirement	1884

A.8.4	E-UTRAN TDD - TDD Inter-frequency Measurements	1884

A.8.4.1	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	1884

A.8.4.1.1	Test Purpose and Environment	1884

A.8.4.1.2	Test Requirements	1886

A.8.4.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells	1886

A.8.4.2.1	Test Purpose and Environment	1886

A.8.4.2.2	Test Requirements	1889

A.8.4.3	E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used	1889

A.8.4.3.1	Test Purpose and Environment	1889

A.8.4.3.2	Test Requirements	1891

A.8.4.4	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1892

A.8.4.4.1	Test Purpose and Environment	1892

A.8.4.4.2	Test Requirements	1894

A.8.4.5	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1895

A.8.4.5.1	Test Purpose and Environment	1895

A.8.4.5.2	Test Requirements	1897

A.8.4.6	E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0	1897

A.8.4.6.1	Test Purpose and Environment	1897

A.8.4.6.2	Test Requirements	1898

A.8.4.7	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for Increased Carrier Monitoring without Reduced Performance Group	1898

A.8.4.7.1	Test Purpose and Environment	1898

A.8.4.7.2	Test Requirements	1902

A.8.4.8	TDD-TDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	1902

A.8.4.8.1	Test Purpose and Environment	1902

A.8.4.8.2	Test Requirements	1905

A.8.4.9	TDD-TDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	1905

A.8.4.9.1	Test Purpose and Environment	1905

A.8.4.9.2	Test Requirements	1909

A.8.4.10	E-UTRAN TDD-TDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	1909

A.8.4.10.1	Test Purpose and Environment	1909

A.8.4.10.2	Test Requirements	1911

A.8.4.11	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells with burst gap	1911

A.8.4.11.1	Test Purpose and Environment	1911

A.8.4.11.2	Test Requirement	1913

A.8.4.12	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	1913

A.8.4.12.1	Test Purpose and Environment	1913

A.8.4.12.2	Test Requirement	1915

A.8.4.13	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	1916

A.8.4.13.1	Test Purpose and Environment	1916

A.8.4.13.2	Test Requirement	1917

A.8.4.14	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	1918

A.8.4.14.1	Test Purpose and Environment	1918

A.8.4.14.2	Test Requirement	1920

A.8.4.15	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	1920

A.8.4.15.1	Test Purpose and Environment	1920

A.8.4.15.2	Test Requirement	1923

A.8.5	E-UTRAN FDD - UTRAN FDD Measurements	1923

A.8.5.1	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	1923

A.8.5.1.1	Test Purpose and Environment	1923

A.8.5.1.2	Test Requirements	1925

A.8.5.2	E-UTRAN FDD - UTRAN FDD SON ANR cell search reporting under AWGN propagation conditions	1925

A.8.5.2.1	Test Purpose and Environment	1925

A.8.5.2.2	Test Requirements	1927

A.8.5.3	E-UTRAN FDD-UTRAN FDD event triggered reporting when DRX is used under fading propagation conditions	1927

A.8.5.3.1	Test Purpose and Environment	1927

A.8.5.3.2	Test Requirements	1930

A.8.5.4	E-UTRAN FDD - UTRAN FDD enhanced cell identification under AWGN propagation conditions	1930

A.8.5.4.1	Test Purpose and Environment	1930

A.8.5.4.2	Test Requirements	1932

A.8.5.5	E- UTRAN FDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	1932

A.8.5.5.1	Test Purpose and Environment	1932

A.8.5.5.2	Test Requirements	1935

A.8.5.6	E-UTRAN FDD - UTRAN FDD event triggered reporting without measurement gaps under AWGN propagation conditions	1935

A.8.5.6.1	Test Purpose and Environment	1935

A.8.5.6.2	Test Requirements	1936

A.8.5.7	E-UTRAN FDD - UTRAN FDD Event Triggered Reporting under Fading Propagation Conditions for 5 MHz Bandwidth	1937

A.8.5.7.1	Test Purpose and Environment	1937

A.8.5.7.2	Test Requirements	1937

A.8.5.8	E-UTRA FDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	1937

A.8.5.8.1	Test Purpose and Environment	1937

A.8.5.8.2	Test Requirements	1940

A.8.6	E-UTRAN TDD - UTRAN FDD Measurements	1940

A.8.6.1	E-UTRAN TDD - UTRAN FDD event triggered reporting under fading propagation conditions	1940

A.8.6.1.1	Test Purpose and Environment	1940

A.8.6.1.2	Test Requirements	1942

A.8.6.2	E- UTRAN TDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	1942

A.8.6.2.1	Test Purpose and Environment	1942

A.8.6.2.2	Test Requirements	1945

A.8.6.3	E-UTRA TDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	1945

A.8.6.3.1	Test Purpose and Environment	1945

A.8.6.3.2	Test Requirements	1948

A.8.7	E-UTRAN TDD – UTRAN TDD Measurements	1948

A.8.7.1	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	1948

A.8.7.1.1	Test Purpose and Environment	1948

A.8.7.1.1.1	Void	1948

A.8.7.1.1.2	1.28 Mcps TDD option	1948

A.8.7.1.1.3	Void	1950

A.8.7.1.2	Test Requirements	1950

A.8.7.1.2.1	Void	1950

A.8.7.1.2.2	1.28 Mcps TDD option	1950

A.8.7.1.2.3	Void	1950

A.8.7.2	E-UTRAN TDD-UTRAN TDD cell search when DRX is used under fading propagation conditions	1950

A.8.7.2.1	Test Purpose and Environment	1950

A.8.7.2.2	Test Requirements	1953

A.8.7.3	E-UTRAN TDD - UTRAN TDD SON ANR cell search reporting in AWGN propagation conditions	1954

A.8.7.3.1	Test Purpose and Environment	1954

A.8.7.3.2	Test Parameters	1954

A.8.7.3.3	Test Requirements	1955

A.8.7.4	E-UTRAN TDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	1956

A.8.7.4.1	Test Purpose and Environment	1956

A.8.7.4.2	Test Requirements	1958

A.8.7.5	E-UTRA TDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	1958

A.8.7.5.1	Test Purpose and Environment	1958

A.8.7.5.2	Test Requirements	1960

A.8.7A	TBD	1960

A.8.7A.1	E-UTRA FDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	1960

A.8.7A.1.1	Test Purpose and Environment	1960

A.8.7A.1.2	Test Requirements	1963

A.8.8	E-UTRAN FDD – GSM Measurements	1963

A.8.8.1	E-UTRAN FDD – GSM event triggered reporting in AWGN	1963

A.8.8.1.1	Test Purpose and Environment	1963

A.8.8.1.2	Test Requirements	1965

A.8.8.2	E-UTRAN FDD-GSM event triggered reporting when DRX is used in AWGN	1965

A.8.8.2.1	Test Purpose and Environment	1965

A.8.8.2.2	Test Requirements	1967

A.8.8.3	E-UTRAN FDD – GSM event triggered reporting in AWGN with enhanced BSIC identification	1968

A.8.8.3.1	Test Purpose and Environment	1968

A.8.8.3.2	Test Requirements	1969

A.8.9	E-UTRAN FDD - UTRAN TDD measurements	1970

A.8.9.1	E-UTRAN FDD - UTRAN TDD event triggered reporting in fading propagation conditions	1970

A.8.9.1.1	Test Purpose and Environment	1970

A.8.9.1.2	Test Requirements	1971

A.8.9.2	E-UTRAN FDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	1972

A.8.9.2.1	Test Purpose and Environment	1972

A.8.9.2.2	Test Requirements	1974

A.8.10	E-UTRAN TDD – GSM Measurements	1974

A.8.10.1	E-UTRAN TDD – GSM event triggered reporting in AWGN	1974

A.8.10.1.1	Test Purpose and Environment	1974

A.8.10.1.2	Test Requirements	1975

A.8.10.2	E-UTRAN TDD-GSM event triggered reporting when DRX is used in AWGN	1976

A.8.10.2.1	Test Purpose and Environment	1976

A.8.10.2.2	Test Requirements	1978

A.8.11	Monitoring of Multiple Layers	1978

A.8.11.1	Multiple E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions	1978

A.8.11.1.1	Test Purpose and Environment	1978

A.8.11.1.2	Test Requirements	1980

A.8.11.2	E-UTRAN TDD – E-UTRAN TDD and E-UTRAN TDD Inter-frequency event triggered reporting under fading propagation conditions	1981

A.8.11.2.1	Test Purpose and Environment	1981

A.8.11.2.2	Test Requirements	1982

A.8.11.3	E-UTRAN FDD-FDD Inter-frequency and UTRAN FDD event triggered reporting under fading propagation conditions	1982

A.8.11.3.1	Test Purpose and Environment	1982

A.8.11.3.2	Test Requirements	1985

A.8.11.4	InterRAT E-UTRA TDD to E-UTRA TDD and UTRA TDD cell search test case	1985

A.8.11.4.1	Test Purpose and Environment	1985

A.8.11.4.2	Test Requirements	1988

A.8.11.5	Combined E-UTRAN FDD – E-UTRA FDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	1988

A.8.11.5.1	Test Purpose and Environment	1988

A.8.11.5.2	Test Requirements	1990

A.8.11.6	Combined E-UTRAN TDD – E-UTRA TDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	1991

A.8.11.6.1	Test Purpose and Environment	1991

A.8.11.6.2	Test Requirements	1993

A.8.12	RSTD Intra-frequency Measurements	1994

A.8.12.1	E-UTRAN FDD intra-frequency RSTD measurement reporting delay test case	1994

A.8.12.1.1	Test Purpose and Environment	1994

A.8.12.1.2	Test Requirements	1997

A.8.12.1.2A	Test Requirements for UE Category 1bis	1998

A.8.12.2	E-UTRAN TDD intra-frequency RSTD measurement reporting delay test case	1998

A.8.12.2.1	Test Purpose and Environment	1998

A.8.12.2.2	Test Requirements	2002

A.8.12.2.2A	Test Requirements for UE Category 1bis	2003

A.8.12.3	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A	2003

A.8.12.3.1	Test Purpose and Environment	2003

A.8.12.3.2	Test Requirements	2007

A.8.12.4	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A	2008

A.8.12.4.1	Test Purpose and Environment	2008

A.8.12.4.2	Test Requirements	2011

A.8.12.5	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A	2012

A.8.12.5.1	Test Purpose and Environment	2012

A.8.12.5.2	Test Requirements	2015

A.8.12.6	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B	2016

A.8.12.6.1	Test Purpose and Environment	2016

A.8.12.6.2	Test Requirements	2019

A.8.12.7	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B	2020

A.8.12.7.1	Test Purpose and Environment	2020

A.8.12.7.2	Test Requirements	2023

A.8.12.8	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B	2024

A.8.12.8.1	Test Purpose and Environment	2024

A.8.12.8.2	Test Requirements	2027

A.8.12.9	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2028

A.8.12.9.1	Test Purpose and Environment	2028

A.8.12.9.2	Test Requirements	2031

A.8.12.10	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2032

A.8.12.10.1	Test Purpose and Environment	2032

A.8.12.10.2	Test Requirements	2035

A.8.12.11	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2036

A.8.12.11.1	Test Purpose and Environment	2036

A.8.12.11.2	Test Requirements	2040

A.8.12.12	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2041

A.8.12.12.1	Test Purpose and Environment	2041

A.8.12.12.2	Test Requirements	2044

A.8.12.13	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2045

A.8.12.13.1	Test Purpose and Environment	2045

A.8.12.13.2	Test Requirements	2048

A.8.12.14	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2049

A.8.12.14.1	Test Purpose and Environment	2049

A.8.12.14.2	Test Requirements	2053

A.8.13	RSTD Inter-frequency Measurements	2054

A.8.13.1	E-UTRAN FDD-FDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2054

A.8.13.1.1	Test Purpose and Environment	2054

A.8.13.1.2	Test Requirements	2057

A.8.13.1.2A	Test Requirements for UE Category 1bis	2058

A.8.13.2	E-UTRAN TDD-TDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2058

A.8.13.2.1	Test Purpose and Environment	2058

A.8.13.2.2	Test Requirements	2062

A.8.13.2.2A	Test Requirements for UE Category 1bis	2063

A.8.13.3	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A	2063

A.8.13.3.1	Test Purpose and Environment	2063

A.8.13.3.2	Test Requirements	2067

A.8.13.4	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A	2068

A.8.13.4.1	Test Purpose and Environment	2068

A.8.13.4.2	Test Requirements	2071

A.8.13.5	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A	2072

A.8.13.5.1	Test Purpose and Environment	2072

A.8.13.5.2	Test Requirements	2076

A.8.13.6	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B	2077

A.8.13.6.1	Test Purpose and Environment	2077

A.8.13.6.2	Test Requirements	2080

A.8.13.7	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B	2081

A.8.13.7.1	Test Purpose and Environment	2081

A.8.13.7.2	Test Requirements	2084

A.8.13.8	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B	2085

A.8.13.8.1	Test Purpose and Environment	2085

A.8.13.8.2	Test Requirements	2089

A.8.13.9	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2090

A.8.13.9.1	Test Purpose and Environment	2090

A.8.13.9.2	Test Requirements	2094

A.8.13.10	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2095

A.8.13.10.1	Test Purpose and Environment	2095

A.8.13.10.2	Test Requirements	2099

A.8.13.11	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2100

A.8.13.11.1	Test Purpose and Environment	2100

A.8.13.11.2	Test Requirements	2104

A.8.13.12	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2105

A.8.13.12.1	Test Purpose and Environment	2105

A.8.13.12.2	Test Requirements	2109

A.8.13.13	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2110

A.8.13.13.1	Test Purpose and Environment	2110

A.8.13.13.2	Test Requirements	2114

A.8.13.14	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2115

A.8.13.14.1	Test Purpose and Environment	2115

A.8.13.14.2	Test Requirements	2119

A.8.14	E-UTRAN TDD - FDD Inter-frequency Measurements	2120

A.8.14.1	E-UTRAN TDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2120

A.8.14.1.1	Test Purpose and Environment	2120

A.8.14.1.2	Test Requirements	2122

A.8.14.2	E-UTRAN TDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2122

A.8.14.2.1	Test Purpose and Environment	2122

A.8.14.2.2	Test Requirements	2125

A.8.14.3	E-UTRAN TDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2125

A.8.14.3.1	Test Purpose and Environment	2125

A.8.14.3.2	Test Requirements	2127

A.8.15	E-UTRAN FDD - TDD Inter-frequency Measurements	2128

A.8.15.1	E-UTRAN FDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2128

A.8.15.1.1	Test Purpose and Environment	2128

A.8.15.1.2	Test Requirements	2129

A.8.15.2	E-UTRAN FDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2129

A.8.15.2.1	Test Purpose and Environment	2129

A.8.15.2.2	Test Requirements	2132

A.8.15.3	E-UTRAN FDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2132

A.8.15.3.1	Test Purpose and Environment	2132

A.8.15.3.2	Test Requirements	2134

A.8.16	E-UTRAN Carrier Aggregation Measurements	2135

A.8.16.1	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX	2135

A.8.16.1.1	Test Purpose and Environment	2135

A.8.16.1.2	Test Requirements	2137

A.8.16.2	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX	2137

A.8.16.2.1	Test Purpose and Environment	2137

A.8.16.2.2	Test Requirements	2139

A.8.16.3	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2139

A.8.16.3.1	Test Purpose and Environment	2139

A.8.16.3.2	Test Requirements	2141

A.8.16.3A	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with network controlled PCell interruption in non-DRX	2142

A.8.16.3A.1	Test Purpose and Environment	2142

A.8.16.3A.2	Test Requirements	2143

A.8.16.4	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell  with PCell interruption in non-DRX	2144

A.8.16.4.1	Test Purpose and Environment	2144

A.8.16.4.2	Test Requirements	2145

A.8.16.4A	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2146

A.8.16.4A.1	Test Purpose and Environment	2146

A.8.16.4A.2	Test Requirements	2148

A.8.16.5	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2149

A.8.16.5.1	Test Purpose and Environment	2149

A.8.16.5.2	Test Requirements	2149

A.8.16.6	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2149

A.8.16.6.1	Test Purpose and Environment	2149

A.8.16.6.2	Test Requirements	2150

A.8.16.7	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2150

A.8.16.7.1	 Test Purpose and Environment	2150

A.8.16.7.2	 Test Requirements	2151

A.8.16.8	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2151

A.8.16.8.1	 Test Purpose and Environment	2151

A.8.16.8.2	 Test Requirements	2152

A.8.16.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2152

A.8.16.9.1	Test Purpose and Environment	2152

A.8.16.9.2	Test Requirements	2153

A.8.16.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2153

A.8.16.10.1	Test Purpose and Environment	2153

A.8.16.10.2	Test Requirements	2154

A.8.16.11	E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2154

A.8.16.11.1	 Test Purpose and Environment	2154

A.8.16.11.2	 Test Requirements	2155

A.8.16.12	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2155

A.8.16.12.1	 Test Purpose and Environment	2155

A.8.16.12.2	 Test Requirements	2155

A.8.16.13	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 5MHz +5 MHz bandwidth	2155

A.8.16.13.1	Test Purpose and Environment	2155

A.8.16.13.2	Test Requirements	2156

A.8.16.14	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 5 MHz +5 MHz bandwidth	2156

A.8.16.14.1	Test Purpose and Environment	2156

A.8.16.14.2	Test Requirements	2157

A.8.16.15	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5 +5 MHz bandwidth	2157

A.8.16.15.1	 Test Purpose and Environment	2157

A.8.16.7.2	 Test Requirements	2157

A.8.16.16	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5+5 MHz bandwidth	2157

A.8.16.16.1	 Test Purpose and Environment	2157

A.8.16.16.2	 Test Requirements	2158

A.8.16.17	E-UTRAN FDD activation and deactivation of known SCell in non-DRX	2158

A.8.16.17.1	Test Purpose and Environment	2158

A.8.16.17.2	Test Requirements	2160

A.8.16.17A	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 20MHz	2161

A.8.16.17A.1	Test Purpose and Environment	2161

A.8.16.17A.2	Test Requirements	2161

A.8.16.17B	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2161

A.8.16.17B.1	Test Purpose and Environment	2161

A.8.16.17B.2	Test Requirements	2162

A.8.16.17C	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2162

A.8.16.17C.1	Test Purpose and Environment	2162

A.8.16.17C.2	Test Requirements	2162

A.8.16.18	E-UTRAN TDD activation and deactivation of known SCell in non-DRX	2162

A.8.16.18.1	Test Purpose and Environment	2162

A.8.16.18.2	Test Requirements	2164

A.8.16.18A	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz	2165

A.8.16.18A.1	Test Purpose and Environment	2165

A.8.16.18A.2	Test Requirements	2165

A.8.16.18B	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2165

A.8.16.18B.1	Test Purpose and Environment	2165

A.8.16.18B.2	Test Requirements	2166

A.8.16.18C	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2166

A.8.16.18C.1	Test Purpose and Environment	2166

A.8.16.18C.2	Test Requirements	2166

A.8.16.18D	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz + 10MHz	2166

A.8.16.18D.1	Test Purpose and Environment	2166

A.8.16.18D.2	Test Requirements	2167

A.8.16.19	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX	2167

A.8.16.19.1	Test Purpose and Environment	2167

A.8.16.19.2	Test Requirements	2169

A.8.16.19A	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2169

A.8.16.19A.1	Test Purpose and Environment	2169

A.8.16.19A.2	Test Requirements	2170

A.8.16.19B 	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2170

A.8.16.19B.1	Test Purpose and Environment	2170

A.8.16.19B.2	Test Requirements	2170

A.8.16.19C	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2170

A.8.16.19C.1	Test Purpose and Environment	2170

A.8.16.19C.2	Test Requirements	2171

A.8.16.20	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX	2171

A.8.16.20.1	Test Purpose and Environment	2171

A.8.16.20.2	Test Requirements	2173

A.8.16.20A	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2174

A.8.16.20A.1	Test Purpose and Environment	2174

A.8.16.20A.2	Test Requirements	2174

A.8.16.20B	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2174

A.8.16.20B.1	Test Purpose and Environment	2174

A.8.16.20B.2	Test Requirements	2175

A.8.16.20C	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2175

A.8.16.20C.1	Test Purpose and Environment	2175

A.8.16.20C.2	Test Requirements	2175

A.8.16.20D	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz + 10MHz	2175

A.8.16.20D.1	Test Purpose and Environment	2175

A.8.16.20D.2	Test Requirements	2176

A.8.16.21	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20MHz+10MHz	2176

A.8.16.21.1	Test Purpose and Environment	2176

A.8.16.21.2	Test Requirements	2178

A.8.16.22	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 20MHz+10MHz	2178

A.8.16.22.1	Test Purpose and Environment	2178

A.8.16.22.2	Test Requirements	2180

A.8.16.23	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in FDD	2180

A.8.16.23.1	Test Purpose and Environment	2180

A.8.16.23.2	Test Requirements	2183

A.8.16.24	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in TDD	2183

A.8.16.24.1	Test Purpose and Environment	2183

A.8.16.24.2	Test Requirements	2186

A.8.16.25	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in FDD	2186

A.8.16.25.1	Test Purpose and Environment	2186

A.8.16.25.2	Test Requirements	2188

A.8.16.26	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in TDD	2188

A.8.16.26.1	Test Purpose and Environment	2188

A.8.16.26.2	Test Requirements	2191

A.8.16.27	3 DL PCell in FDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2191

A.8.16.27.1	Test Purpose and Environment	2191

A.8.16.27.2	Test Requirements	2195

A.8.16.28	3 DL PCell in TDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2195

A.8.16.28.1	Test Purpose and Environment	2195

A.8.16.28.2	Test Requirements	2199

A.8.16.29	3 DL FDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2199

A.8.16.29.1	Test Purpose and Environment	2199

A.8.16.29.2	Test Requirements	2203

A.8.16.30	3 DL TDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2203

A.8.16.30.1	Test Purpose and Environment	2203

A.8.16.30.2	Test Requirements	2207

A.8.16.31	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in FDD	2207

A.8.16.31.1	Test Purpose and Environment	2207

A.8.16.31.2	Test Requirements	2211

A.8.16.32	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in TDD	2211

A.8.16.32.1	Test Purpose and Environment	2211

A.8.16.32.2	Test Requirements	2215

A.8.16.33	E-UTRAN FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2215

A.8.16.33.1	Test Purpose and Environment	2215

A.8.16.33.2	Test Requirements	2219

A.8.16.34	E-UTRAN TDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2219

A.8.16.34.1	Test Purpose and Environment	2219

A.8.16.34.2	Test Requirements	2223

A.8.16.35	3 DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX	2223

A.8.16.35.1	Test Purpose and Environment	2223

A.8.16.35.2	Test Requirements	2225

A.8.16.36	3 DL PCell in TDD CA Activation and Deactivation of Known SCell in Non-DRX	2226

A.8.16.36.1	Test Purpose and Environment	2226

A.8.16.36.2	Test Requirements	2228

A.8.16.37	3 DL FDD CA activation and deactivation of known SCell in non-DRX	2229

A.8.16.37.1	Test Purpose and Environment	2229

A.8.16.37.2	Test Requirements	2231

A.8.16.38	3 DL TDD CA activation and deactivation of known SCell in non-DRX	2232

A.8.16.38.1	Test Purpose and Environment	2232

A.8.16.38.2	Test Requirements	2234

A.8.16.39	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in FDD	2235

A.8.16.39.1	Test Purpose and Environment	2235

A.8.16.39.2	Test Requirements	2238

A.8.16.40	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in TDD	2238

A.8.16.40.1	Test Purpose and Environment	2238

A.8.16.40.2	Test Requirements	2241

A.8.16.41	3 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2241

A.8.16.41.1	Test Purpose and Environment	2241

A.8.16.41.2	Test Requirements	2243

A.8.16.42	3 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2244

A.8.16.42.1	Test Purpose and Environment	2244

A.8.16.42.2	Test Requirements	2246

A.8.16.43	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in FDD	2247

A.8.16.43.1	Test Purpose and Environment	2247

A.8.16.43.2	Test Requirements	2250

A.8.16.44	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in FDD	2250

A.8.16.44.1	Test Purpose and Environment	2250

A.8.16.44.2	Test Requirements	2253

A.8.16.45	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in TDD	2253

A.8.16.45.1	Test Purpose and Environment	2253

A.8.16.45.2	Test Requirements	2256

A.8.16.46	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in TDD	2256

A.8.16.46.1	Test Purpose and Environment	2256

A.8.16.46.2	Test Requirements	2258

A.8.16.47	2DL/2UL FDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2259

A.8.16.47.1	Test Purpose and Environment	2259

A.8.16.47.2	Test Requirements	2261

A.8.16.48	2DL/2UL TDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2262

A.8.16.48.1	Test Purpose and Environment	2262

A.8.16.48.2	Test Requirements	2264

A.8.16.49	2DL/2UL TDD-FDD CA (FDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2265

A.8.16.49.1	Test Purpose and Environment	2265

A.8.16.49.2	Test Requirements	2268

A.8.16.50	2DL/2UL TDD-FDD CA (TDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2268

A.8.16.50.1	Test Purpose and Environment	2268

A.8.16.50.2	Test Requirements	2270

A.8.16.51	E-UTRAN 4 DL FDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2271

A.8.16.51.1	Test Purpose and Environment	2271

A.8.16.51.2	Test Requirements	2275

A.8.16.52	E-UTRAN 4 DL TDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2275

A.8.16.52.1	Test Purpose and Environment	2275

A.8.16.52.2	Test Requirements	2279

A.8.16.53	4 DL PCell in FDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2279

A.8.16.53.1	Test Purpose and Environment	2279

A.8.16.53.2	Test Requirements	2283

A.8.16.54	4 DL PCell in TDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2283

A.8.16.54.1	Test Purpose and Environment	2283

A.8.16.54.2	Test Requirements	2287

A.8.16.55	E-UTRAN FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2287

A.8.16.55.1	Test Purpose and Environment	2287

A.8.16.55.2	Test Requirements	2292

A.8.16.56	E-UTRAN TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2292

A.8.16.56.1	Test Purpose and Environment	2292

A.8.16.56.2	Test Requirements	2297

A.8.16.57	E-UTRAN FDD 4DL CA activation and deactivation of know SCell in non-DRX	2297

A.8.16.57.1	Test Purpose and Environment	2297

A.8.16.57.2	Test Requirements	2299

A.8.16.58	E-UTRAN TDD 4DL CA activation and deactivation of know SCell in non-DRX	2300

A.8.16.58.1	Test Purpose and Environment	2300

A.8.16.58.2	Test Requirements	2302

A.8.16.59	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2303

A.8.16.59.1	Test Purpose and Environment	2303

A.8.16.59.2	Test Requirements	2306

A.8.16.60	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2307

A.8.16.60.1	Test Purpose and Environment	2307

A.8.16.60.2	Test Requirements	2310

A.8.16.61	E-UTRAN FDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2311

A.8.16.61.1	Test Purpose and Environment	2311

A.8.16.61.2	Test Requirements	2314

A.8.16.62	E-UTRAN TDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2314

A.8.16.62.1	Test Purpose and Environment	2314

A.8.16.62.2	Test Requirements	2316

A.8.16.63	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2317

A.8.16.63.1	Test Purpose and Environment	2317

A.8.16.63.2	Test Requirements	2320

A.8.16.64	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2321

A.8.16.64.1	Test Purpose and Environment	2321

A.8.16.64.2	Test Requirements	2325

A.8.16.65	5 DL FDD-TDD with PCell in FDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2325

A.8.16.65.1	Test Purpose and Environment	2325

A.8.16.65.2	Test Requirements	2330

A.8.16.66	5 DL FDD-TDD with PCell in TDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2330

A.8.16.66.1	Test Purpose and Environment	2330

A.8.16.66.2	Test Requirements	2335

A.8.16.67	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of Unknown SCell in non-DRX	2335

A.8.16.67.1	Test Purpose and Environment	2335

A.8.16.67.2	Test Requirements	2340

A.8.16.68	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of Unknown SCell in non-DRX	2340

A.8.16.68.1	Test Purpose and Environment	2340

A.8.16.68.2	Test Requirements	2345

A.8.16.69	5 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2345

A.8.16.69.1	Test Purpose and Environment	2345

A.8.16.69.2	Test Requirements	2348

A.8.16.70	5 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2348

A.8.16.70.1	Test Purpose and Environment	2348

A.8.16.70.2	Test Requirements	2351

A.8.16.71	5 DL FDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2351

A.8.16.71.1	Test Purpose and Environment	2351

A.8.16.71.2	Test Requirements	2355

A.8.16.72	5 DL TDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2355

A.8.16.72.1	Test Purpose and Environment	2355

A.8.16.72.2	Test Requirements	2359

A.8.16.73	5 DL FDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2359

A.8.16.73.1	Test Purpose and Environment	2359

A.8.16.73.2	Test Requirements	2364

A.8.16.74	5 DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2364

A.8.16.74.1	Test Purpose and Environment	2364

A.8.16.74.2	Test Requirements	2369

A.8.16.75	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of known SCell in non-DRX	2369

A.8.16.75.1	Test Purpose and Environment	2369

A.8.16.75.2	Test Requirements	2374

A.8.16.76	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of known SCell in non-DRX	2374

A.8.16.76.1	Test Purpose and Environment	2374

A.8.16.76.2	Test Requirements	2379

A.8.16.77	5 DL FDD CA activation and deactivation of know SCell in non-DRX	2379

A.8.16.77.1	Test Purpose and Environment	2379

A.8.16.77.2	Test Requirements	2382

A.8.16.78	5 DL TDD CA activation and deactivation of know SCell in non-DRX	2382

A.8.16.78.1	Test Purpose and Environment	2382

A.8.16.78.2	Test Requirements	2385

A.8.16.79	E-UTRAN PCell in FDD FDD-TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2386

A.8.16.79.1	Test Purpose and Environment	2386

A.8.16.79.2	Test Requirements	2391

A.8.16.80	E-UTRAN PCell in TDD TDD-FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2391

A.8.16.80.1	Test Purpose and Environment	2391

A.8.16.80.2	Test Requirements	2396

A.8.16.81	E-UTRAN PCell in FDD FDD-TDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2396

A.8.16.81.1	Test Purpose and Environment	2396

A.8.16.81.2	Test Requirements	2401

A.8.16.82	E-UTRAN PCell in TDD TDD-FDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2401

A.8.16.82.1	Test Purpose and Environment	2401

A.8.16.82.2	Test Requirements	2406

A.8.16.83	3 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2406

A.8.16.83.1	Test Purpose and Environment	2406

A.8.16.83.2	Test Requirements	2411

A.8.16.84	3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2411

A.8.16.84.1	Test Purpose and Environment	2411

A.8.16.84.2	Test Requirements	2417

A.8.16.85	3 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2417

A.8.16.85.1	Test Purpose and Environment	2417

A.8.16.85.2	Test Requirements	2420

A.8.16.86	3 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2420

A.8.16.86.1	Test Purpose and Environment	2420

A.8.16.86.2	Test Requirements	2423

A.8.16.87	4 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2423

A.8.16.87.1	Test Purpose and Environment	2423

A.8.16.87.2	Test Requirements	2428

A.8.16.88	4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2428

A.8.16.88.1	Test Purpose and Environment	2428

A.8.16.88.2	Test Requirements	2436

A.8.16.89	4 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2436

A.8.16.89.1	Test Purpose and Environment	2436

A.8.16.89.2	Test Requirements	2441

A.8.16.90	4 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2441

A.8.16.90.1	Test Purpose and Environment	2441

A.8.16.90.2	Test Requirements	2446

A.8.16.91	5 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2446

A.8.16.91.1	Test Purpose and Environment	2446

A.8.16.91.2	Test Requirements	2452

A.8.16.92	5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2452

A.8.16.92.1	Test Purpose and Environment	2452

A.8.16.92.2	Test Requirements	2460

A.8.16.93	5 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2460

A.8.16.93.1	Test Purpose and Environment	2460

A.8.16.93.2	Test Requirements	2465

A.8.16.94	5 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2465

A.8.16.94.1	Test Purpose and Environment	2465

A.8.16.94.2	Test Requirements	2470

A.8.16.95	6 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2470

A.8.16.95.1	Test Purpose and Environment	2470

A.8.16.95.2	Test Requirements	2478

A.8.16.96	6 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2478

A.8.16.96.1	Test Purpose and Environment	2478

A.8.16.96.2	Test Requirements	2485

A.8.16.97	6 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2485

A.8.16.97.1	Test Purpose and Environment	2485

A.8.16.97.2	Test Requirements	2490

A.8.16.98	6 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2490

A.8.16.98.1	Test Purpose and Environment	2490

A.8.16.98.2	Test Requirements	2496

A.8.16.99	7 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2496

A.8.16.99.1	Test Purpose and Environment	2496

A.8.16.99.2	Test Requirements	2503

A.8.16.100	7 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2503

A.8.16.100.1	Test Purpose and Environment	2503

A.8.16.100.2	Test Requirements	2511

A.8.16.101	7 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2511

A.8.16.101.1	Test Purpose and Environment	2511

A.8.16.101.2	Test Requirements	2518

A.8.16.102	7 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2518

A.8.16.102.1	Test Purpose and Environment	2518

A.8.16.102.2	Test Requirements	2526

A.8.16.103	Hibernation and Activation of Known SCell in Non-DRX with generic duplex modes	2526

A.8.16.103.1	Test Purpose and Environment	2526

A.8.16.103.2	Test Requirements	2530

A.8.16.104	Hibernation and Activation of Unknown SCell in Non-DRX with generic duplex modes	2531

A.8.16.104.1	Test Purpose and Environment	2531

A.8.16.104.2	Test Requirements	2534

A.8.16.105	Idle Mode measurements of inter-frequency CA candidate cells for early reporting	2535

A.8.16.105.1	Test Purpose and Environment	2535

A.8.16.105.2	Test Requirements	2539

A.8.16.106	Direct Activation of Known SCell in Non-DRX with generic duplex modes	2540

A.8.16.106.1	Test Purpose and Environment	2540

A.8.16.106.2	Test Requirements	2543

A.8.17	RSTD Measurements for E-UTRAN Carrier Aggregation	2543

A.8.17.1	E-UTRAN FDD RSTD measurement reporting delay test case	2543

A.8.17.1.1	Test Purpose and Environment	2543

A.8.17.1.2	Test Requirements	2549

A.8.17.2	E-UTRAN TDD RSTD measurement reporting delay test case	2549

A.8.17.2.1	Test Purpose and Environment	2549

A.8.17.2.2	Test Requirements	2555

A.8.17.3	E-UTRAN FDD RSTD Measurement Reporting Test Case for 20 MHz	2555

A.8.17.3.1	Test Purpose and Environment	2555

A.8.17.3.2	Test Requirements	2556

A.8.17.4	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20 MHz	2556

A.8.17.4.1	Test Purpose and Environment	2556

A.8.17.4.2	Test Requirements	2557

A.8.17.5	E-UTRAN FDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2557

A.8.17.5.1	Test Purpose and Environment	2557

A.8.17.5.2	Test Requirements	2558

A.8.17.6	E-UTRAN TDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2558

A.8.17.6.1	Test Purpose and Environment	2558

A.8.17.6.2	Test Requirements	2559

A.8.17.7	E-UTRAN FDD RSTD Measurement Reporting Test Case for 5 + 5 MHz Bandwidth	2560

A.8.17.7.1	Test Purpose and Environment	2560

A.8.17.7.2	Test Requirements	2560

A.8.17.8	E-UTRAN TDD RSTD Measurement Reporting Test Case for 5+5 MHz bandwidth	2561

A.8.17.8.1	Test Purpose and Environment	2561

A.8.17.8.2	Test Requirements	2561

A.8.17.9	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20MHz+10MHz	2562

A.8.17.9.1	Test Purpose and Environment	2562

A.8.17.9.2	Test Requirements	2563

A.8.17.10	E-UTRAN 3 DL FDD CA RSTD Measurement Reporting Delay Test Case	2563

A.8.17.10.1	Test Purpose and Environment	2563

A.8.17.10.2	Test Requirements	2568

A.8.17.11	E-UTRAN 3 DL TDD CA RSTD Measurement Reporting Delay Test Case	2569

A.8.17.11.1	Test Purpose and Environment	2569

A.8.17.11.2	Test Requirements	2575

A.8.18	E-UTRAN TDD – HRPD Measurements	2576

A.8.18.1	E-UTRAN TDD-HRPD event triggered reporting under fading propagation conditions	2576

A.8.18.1.1	Test Purpose and Environment	2576

A.8.18.1.2	Test Requirements	2578

A.8.19	E-UTRAN TDD – CDMA2000 1X Measurements	2578

A.8.19.1	E-UTRAN TDD – CDMA2000 1X event triggered reporting under fading propagation conditions	2578

A.8.19.1.1	Test Purpose and Environment	2578

A.8.19.1.2	Test Requirements	2579

A.8.20	Inter-frequency/RAT Measurements in CA mode	2580

A.8.20.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2580

A.8.20.1.1	Test Purpose and Environment	2580

A.8.20.1.2	Test Requirements	2581

A.8.20.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2581

A.8.20.2.1	Test Purpose and Environment	2582

A.8.20.2.2	Test Requirements	2583

A.8.20.2A	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth.	2584

A.8.20.2A.1	Test Purpose and Environment	2584

A.8.20.2A.2	Test Requirements	2584

A.8.20.2B	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +10 MHz bandwidth.	2584

A.8.20.2B.1	Test Purpose and Environment	2584

A.8.20.2B.2	Test Requirements	2587

A.8.20.3	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	2587

A.8.20.3.1	Test Purpose and Environment	2587

A.8.20.3.2	Test Requirements	2589

A.8.20.4	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	2589

A.8.20.4.1	Test Purpose and Environment	2589

A.8.20.4.1.1	1.28 Mcps TDD option	2589

A.8.20.4.2	Test Requirements	2591

A.8.20.4.2.1	1.28 Mcps TDD option	2591

A.8.20.4A	E-UTRAN TDD with 20 MHz +20 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2591

A.8.20.4A.1	Test Purpose and Environment	2591

A.8.20.4A.1.1	1.28 Mcps TDD option	2591

A.8.20.4A.2	Test Requirements	2591

A.8.20.4A.2.1	1.28 Mcps TDD option	2591

A.8.20.4B	E-UTRAN TDD with 20 MHz +10 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2592

A.8.20.4B.1	Test Purpose and Environment	2592

A.8.20.4B.1.1	1.28 Mcps TDD option	2592

A.8.20.4B.2	Test Requirements	2594

A.8.20.4B.2.1	1.28 Mcps TDD option	2594

A.8.21	CSG Proximity Indication Testing Case for E-UTRAN FDD – FDD Inter frequency	2594

A.8.21.1	Test Purpose and Environment	2594

A.8.21.2 Test Requirements	2597

A.8.22	E-UTRAN Discovery Signal Measurements	2597

A.8.22.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2597

A.8.22.1.1	Test Purpose and Environment	2597

A.8.22.1.2	Test Requirements	2600

A.8.22.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2600

A.8.22.2.1	Test Purpose and Environment	2600

A.8.22.2.2	Test Requirements	2603

A.8.22.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2603

A.8.22.3.1	Test Purpose and Environment	2603

A.8.22.3.2	Test Requirements	2606

A.8.22.4	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2606

A.8.22.4.1	Test Purpose and Environment	2606

A.8.22.4.2	Test Requirements	2609

A.8.22.5	E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2609

A.8.22.5.1	Test Purpose and Environment	2609

A.8.22.5.2	Test Requirements	2612

A.8.22.6	E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2612

A.8.22.6.1	Test Purpose and Environment	2612

A.8.22.6.2	Test Requirements	2615

A.8.22.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2615

A.8.22.7.1	Test Purpose and Environment	2615

A.8.22.7.2	Test Requirements	2618

A.8.22.8	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation condition in DRX based on CSI-RS based discovery signal	2618

A.8.22.8.1	Test Purpose and Environment	2618

A.8.22.8.2	Test Requirements	2621

A.8.22.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2621

A.8.22.9.1	Test Purpose and Environment	2621

A.8.22.9.2	Test Requirements	2623

A.8.22.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2624

A.8.22.10.1	Test Purpose and Environment	2624

A.8.22.10.2	Test Requirements	2626

A.8.22.11	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2627

A.8.22.11.1	Test Purpose and Environment	2627

A.8.22.11.2	Test Requirements	2629

A.8.22.12	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2629

A.8.22.12.1	Test Purpose and Environment	2629

A.8.22.12.2	Test Requirements	2632

A.8.23	E-UTRAN Dual Connectivity Measurements	2632

A.8.23.1	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2632

A.8.23.1.1	Test Purpose and Environment	2632

A.8.23.1.2	Test Requirements	2635

A.8.23.2	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in asynchronous DC	2635

A.8.23.2.1	Test Purpose and Environment	2635

A.8.23.2.2	Test Requirements	2638

A.8.23.3	E-UTRAN TDD-TDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2638

A.8.23.3.1	Test Purpose and Environment	2638

A.8.23.3.2	Test Requirements	2641

A.8.23.4	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2641

A.8.23.4.1	Test Purpose and Environment	2641

A.8.23.4.2	Test Requirements	2644

A.8.23.5	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in asynchronous DC	2644

A.8.23.5.1	Test Purpose and Environment	2644

A.8.23.5.2	Test Requirements	2647

A.8.23.6	E-UTRAN TDD-TDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2647

A.8.23.6.1	Test Purpose and Environment	2647

A.8.23.6.2	Test Requirements	2650

A.8.23.7	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Synchronous DC	2650

A.8.23.7.1	Test Purpose and Environment	2650

A.8.23.7.2	Test Requirements	2652

A.8.23.8	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Asynchronous DC	2653

A.8.23.8.1	Test Purpose and Environment	2653

A.8.23.8.2	Test Requirements	2655

A.8.23.9	E-UTRAN TDD Addition and Release Delay of known PSCell in Synchronous DC	2656

A.8.23.9.1	Test Purpose and Environment	2656

A.8.23.9.2	Test Requirements	2659

A.8.23.10	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2659

A.8.23.10.1	Test Purpose and Environment	2659

A.8.23.10.2	Test Requirements	2662

A.8.23.11	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2662

A.8.23.11.1	Test Purpose and Environment	2662

A.8.23.11.2	Test Requirements	2665

A.8.23.12	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2665

A.8.23.12.1	Test Purpose and Environment	2665

A.8.23.12.2	Test Requirements	2668

A.8.23.13	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2668

A.8.23.13.1	Test Purpose and Environment	2668

A.8.23.13.2	Test Requirements	2671

A.8.23.14	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in FDD	2671

A.8.23.14.1	Test Purpose and Environment	2671

A.8.23.14.2	Test Requirements	2674

A.8.23.15	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in TDD	2674

A.8.23.15.1	Test Purpose and Environment	2674

A.8.23.15.2	Test Requirements	2677

A.8.23.16	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with no DRX in asynchronous DC	2677

A.8.23.16.1	Test Purpose and Environment	2677

A.8.23.16.2	Test Requirements	2678

A.8.23.17	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with DRX in asynchronous DC	2679

A.8.23.17.1	Test Purpose and Environment	2679

A.8.23.17.2	Test Requirements	2681

A.8.23.18	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2681

A.8.23.18.1	Test Purpose and Environment	2681

A.8.23.18.2	Test Requirements	2682

A.8.23.19	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	2683

A.8.23.19.1	Test Purpose and Environment	2683

A.8.23.19.2	Test Requirements	2684

A.8.23.20	E-UTRAN TDD - TDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2685

A.8.23.20.1	Test Purpose and Environment	2685

A.8.23.20.2	Test Requirements	2686

A.8.23.21	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2687

A.8.23.21.1	Test Purpose and Environment	2687

A.8.23.21.2	Test Requirements	2688

A.8.23.22	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	2689

A.8.23.22.1	Test Purpose and Environment	2689

A.8.23.22.2	Test Requirements	2690

A.8.23.23	E-UTRAN TDD - TDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2691

A.8.23.23.1	Test Purpose and Environment	2691

A.8.23.23.2	Test Requirements	2692

A.8.23.24	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	2693

A.8.23.24.1	Test Purpose and Environment	2693

A.8.23.24.2	Test Requirements	2695

A.8.23.25	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in asynchronous DC	2696

A.8.23.25.1	Test Purpose and Environment	2696

A.8.23.25.2	Test Requirements	2698

A.8.23.26	E-UTRAN TDD-TDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	2699

A.8.23.26.1	Test Purpose and Environment	2699

A.8.23.26.2	Test Requirements	2701

A.8.23.27	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	2702

A.8.23.27.1	Test Purpose and Environment	2702

A.8.23.27.2	Test Requirements	2705

A.8.23.28	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in asynchronous DC	2705

A.8.23.28.1	Test Purpose and Environment	2705

A.8.23.28.2	Test Requirements	2708

A.8.23.29	E-UTRAN TDD-TDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	2708

A.8.23.29.1	Test Purpose and Environment	2708

A.8.23.29.2	Test Requirements	2711

A.8.24	Proximity-based Services	2711

A.8.24.1	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	2711

A.8.24.1.1	Test Purpose and Environment	2711

A.8.24.1.2	Test Requirements	2712

A.8.24.2	E-UTRAN TDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	2713

A.8.24.2.1	Test Purpose and Environment	2713

A.8.24.2.2	Test Requirements	2714

A.8.24.3	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Communication	2714

A.8.24.3.1	Test Purpose and Environment	2715

A.8.24.3.2	Test Requirements	2716

A.8.25	E-UTRAN-WLAN Measurements	2717

A.8.25.1	E-UTRAN FDD-WLAN Event Triggered Reporting in non-DRX under AWGN	2717

A.8.25.1.1	Test Purpose and Environment	2717

A.8.25.1.2	Test Requirements	2719

A.8.25.2	E-UTRAN TDD-WLAN Event Triggered Reporting in non-DRX under AWGN	2719

A.8.25.2.1	Test Purpose and Environment	2719

A.8.25.2.2	Test Requirements	2721

A.8.26 Frame Structure 3 (FS3)	2721

A.8.26.1	E-UTRAN FDD-FS3 Activation and deactivation of known FS3 SCell with FDD PCell in non-DRX	2721

A.8.26.1.1	Test Purpose and Environment	2721

A.8.26.1.2	Test Requirements	2723

A.8.26.2	E-UTRAN TDD-FS3 Activation and deactivation of known FS3 SCell with TDD PCell in non-DRX	2724

A.8.26.2.1	Test Purpose and Environment	2724

A.8.26.2.2	Test Requirements	2726

A.8.26.3	E-UTRAN FDD-FS3 Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2727

A.8.26.3.1	Test Purpose and Environment	2727

A.8.26.3.2	Test Requirements	2730

A.8.26.3A	E-UTRAN FDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2730

A.8.26.3A.1	Test Purpose and Environment	2730

A.8.26.3A.2	Test Requirements	2733

A.8.26.4	E-UTRAN TDD-FS3 Event triggered reporting on deactivated FS3 SCell and TDD PCell interruption in non-DRX	2733

A.8.26.4.1	Test Purpose and Environment	2733

A.8.26.4.2	Test Requirements	2736

A.8.26.4A	E-UTRAN TDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2736

A.8.26.4A.1	Test Purpose and Environment	2736

A.8.26.4A.2	Test Requirements	2739

A.8.26.5	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	2739

A.8.26.5.1	Test Purpose and Environment	2739

A.8.26.5.2	Test Requirements	2742

A.8.26.5A	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	2742

A.8.26.5A.1	Test Purpose and Environment	2742

A.8.26.5A.2	Test Requirements	2745

A.8.26.6	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	2745

A.8.26.6.1	Test Purpose and Environment	2745

A.8.26.6.2	Test Requirements	2748

A.8.26.6A	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	2748

A.8.26.6A.1	Test Purpose and Environment	2748

A.8.26.6A.2	Test Requirements	2751

A.8.26.7	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	2751

A.8.26.7.1	Test Purpose and Environment	2751

A.8.26.7.2	Test Requirements	2754

A.8.26.8	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	2754

A.8.26.8.1	Test Purpose and Environment	2754

A.8.26.8.2	Test Requirements	2757

A.8.26.9	E-UTRAN FDD-FS3 Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2757

A.8.26.9.1	Test Purpose and Environment	2757

A.8.26.9.2	Test Requirements	2760

A.8.26.10	E-UTRAN TDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2760

A.8.26.10.1	Test Purpose and Environment	2760

A.8.26.10.2	Test Requirements	2762

A.9	Measurement Performance Requirements	2764

A.9.1	RSRP	2764

A.9.1.1	FDD Intra frequency case	2764

A.9.1.1.1	Test Purpose and Environment	2764

A.9.1.1.2	Test parameters	2764

A.9.1.1.3	Test Requirements	2767

A.9.1.2	TDD Intra frequency case	2767

A.9.1.2.1	Test Purpose and Environment	2767

A.9.1.2.2	Test parameters	2767

A.9.1.2.3	Test Requirements	2768

A.9.1.3	FDD—FDD Inter frequency case	2769

A.9.1.3.1	Test Purpose and Environment	2769

A.9.1.3.2	Test parameters	2769

A.9.1.3.3	Test Requirements	2772

A.9.1.4	TDD—TDD Inter frequency case	2772

A.9.1.4.1	Test Purpose and Environment	2772

A.9.1.4.2	Test parameters	2772

A.9.1.4.3	Test Requirements	2774

A.9.1.5	FDD—TDD Inter frequency case	2775

A.9.1.5.1	Test Purpose and Environment	2775

A.9.1.5.2	Test parameters	2775

A.9.1.5.3	Test Requirements	2776

A.9.1.6	FDD RSRP for E-UTRAN Carrier Aggregation	2777

A.9.1.6.1	Test Purpose and Environment	2777

A.9.1.6.2	Test parameters	2777

A.9.1.6.3	Test Requirements	2780

A.9.1.7	TDD RSRP for E-UTRAN Carrier Aggregation	2780

A.9.1.7.1	Test Purpose and Environment	2780

A.9.1.7.2	Test parameters	2780

A.9.1.7.3	Test Requirements	2783

A.9.1.8	FDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2783

A.9.1.8.1	Test Purpose and Environment	2783

A.9.1.8.2	Test parameters	2783

A.9.1.8.3	Test Requirements	2787

A.9.1.9	TDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2787

A.9.1.9.1	Test Purpose and Environment	2787

A.9.1.9.2	Test parameters	2787

A.9.1.9.3	Test Requirements	2789

A.9.1.10	FDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	2790

A.9.1.10.1	Test Purpose and Environment	2790

A.9.1.10.2	Test parameters	2790

A.9.1.10.3	Test Requirements	2793

A.9.1.11	TDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	2793

A.9.1.11.1	Test Purpose and Environment	2793

A.9.1.11.2	Test parameters	2793

A.9.1.11.3	Test Requirements	2797

A.9.1.12	FDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	2797

A.9.1.12.1	Test Purpose and Environment	2797

A.9.1.12.2	Test parameters	2797

A.9.1.12.3	Test Requirements	2798

A.9.1.13	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	2798

A.9.1.13.1	Test Purpose and Environment	2798

A.9.1.13.2	Test parameters	2798

A.9.1.13.3	Test Requirements	2799

A.9.1.14	FDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2799

A.9.1.14.1	Test Purpose and Environment	2799

A.9.1.14.2	Test parameters	2799

A.9.1.14.3	Test Requirements	2803

A.9.1.15	TDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2803

A.9.1.15.1	Test Purpose and Environment	2803

A.9.1.15.2	Test parameters	2803

A.9.1.15.3	Test Requirements	2805

A.9.1.16	FDD Intra frequency case for 5MHz Bandwidth	2806

A.9.1.16.1	Test Purpose and Environment	2806

A.9.1.16.2	Test parameters	2806

A.9.1.16.3	Test Requirements	2807

A.9.1.17	FDD—FDD Inter frequency case for 5MHz Bandwidth	2807

A.9.1.17.1	Test Purpose and Environment	2807

A.9.1.17.2	Test parameters	2808

A.9.1.17.3	Test Requirements	2809

A.9.1.18	FDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	2809

A.9.1.18.1	Test Purpose and Environment	2809

A.9.1.18.2	Test parameters	2809

A.9.1.18.3	Test Requirements	2810

A.9.1.19	TDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	2811

A.9.1.19.1	Test Purpose and Environment	2811

A.9.1.19.2	Test parameters	2811

A.9.1.19.3	Test Requirements	2811

A.9.1.20	FDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	2811

A.9.1.20.1	Test Purpose and Environment	2811

A.9.1.20.2	Test parameters	2811

A.9.1.20.3	Test Requirements	2813

A.9.1.21	TDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	2813

A.9.1.21.1	Test Purpose and Environment	2813

A.9.1.21.2	Test parameters	2813

A.9.1.21.3	Test Requirements	2813

A.9.1.22	RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	2813

A.9.1.22.1	Test Purpose and Environment	2813

A.9.1.22.2	Test parameters	2814

A.9.1.22.3	Test Requirements	2818

A.9.1.23 RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	2818

A.9.1.23.1	Test Purpose and Environment	2818

A.9.1.23.2	Test parameters	2818

A.9.1.23.3	Test Requirements	2822

A.9.1.24	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz + 10MHz	2822

A.9.1.24.1	Test Purpose and Environment	2822

A.9.1.24.2	Test parameters	2822

A.9.1.24.3	Test Requirements	2823

A.9.1.25	FDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2823

A.9.1.25.1	Test Purpose and Environment	2823

A.9.1.25.2	Test parameters	2823

A.9.1.25.3	Test Requirements	2826

A.9.1.26	TDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2826

A.9.1.26.1	Test Purpose and Environment	2826

A.9.1.26.2	Test parameters	2826

A.9.1.26.3	Test Requirements	2829

A.9.1.27	FDD—FDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2829

A.9.1.27.1	Test Purpose and Environment	2829

A.9.1.27.2	Test parameters	2829

A.9.1.27.3	Test Requirements	2832

A.9.1.28	TDD—TDD inter-frequency absolute and relative  RSRP accuracies in CRS based discovery signal	2832

A.9.1.28.1	Test Purpose and Environment	2832

A.9.1.28.2	Test parameters	2832

A.9.1.28.3	Test Requirements	2835

A.9.1.29	FDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2835

A.9.1.29.1	Test Purpose and Environment	2835

A.9.1.29.2	Test parameters	2835

A.9.1.29.3	Test Requirements	2838

A.9.1.30	TDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2838

A.9.1.30.1	Test Purpose and Environment	2838

A.9.1.30.2	Test parameters	2839

A.9.1.30.3	Test Requirements	2842

A.9.1.31	FDD—FDD inter-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2842

A.9.1.31.1	Test Purpose and Environment	2842

A.9.1.31.2	Test parameters	2842

A.9.1.31.3	Test Requirements	2845

A.9.1.32	TDD—TDD inter-frequency absolute and relative  CSI-RSRP accuracies in CSI-RS based discovery signal	2845

A.9.1.32.1	Test Purpose and Environment	2845

A.9.1.32.2	Test parameters	2846

A.9.1.32.3	Test Requirements	2849

A.9.1.33	FDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	2849

A.9.1.33.1	Test Purpose and Environment	2849

A.9.1.33.2	Test parameters	2849

A.9.1.33.3	Test Requirements	2852

A.9.1.34	TDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	2852

A.9.1.34.1	Test Purpose and Environment	2852

A.9.1.34.2	Test parameters	2852

A.9.1.34.3	Test Requirements	2855

A.9.1.35	FDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	2855

A.9.1.35.1	Test Purpose and Environment	2855

A.9.1.35.2	Test parameters	2855

A.9.1.35.3	Test Requirements	2858

A.9.1.36	TDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	2859

A.9.1.36.1	Test Purpose and Environment	2859

A.9.1.36.2	Test parameters	2859

A.9.1.36.3	Test Requirements	2862

A.9.1.37	3 DL PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	2862

A.9.1.37.1	Test Purpose and Environment	2862

A.9.1.37.2	Test parameters	2862

A.9.1.37.3	Test Requirements	2865

A.9.1.38	3 DL PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	2866

A.9.1.38.1	Test Purpose and Environment	2866

A.9.1.38.2	Test parameters	2866

A.9.1.38.3	Test Requirements	2869

A.9.1.39	3 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2870

A.9.1.39.1	Test Purpose and Environment	2870

A.9.1.39.2	Test parameters	2870

A.9.1.39.3	Test Requirements	2876

A.9.1.40	3 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2876

A.9.1.40.1	Test Purpose and Environment	2876

A.9.1.40.2	Test parameters	2877

A.9.1.40.3	Test Requirements	2883

A.9.1.41	FD-FDD RSRP Intra frequency case for UE category 0	2883

A.9.1.41.1	Test Purpose and Environment	2883

A.9.1.41.2	Test parameters	2883

A.9.1.41.3	Test Requirements	2886

A.9.1.42	HD-FDD RSRP Intra frequency case for UE category 0	2886

A.9.1.42.1	Test Purpose and Environment	2886

A.9.1.42.2	Test parameters	2886

A.9.1.42.3	Test Requirements	2889

A.9.1.43	TDD RSRP Intra frequency case for UE category 0	2889

A.9.1.43.1	Test Purpose and Environment	2889

A.9.1.43.2	Test parameters	2889

A.9.1.43.3	Test Requirements	2890

A.9.1.44	4 DL CA PCell in FDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	2891

A.9.1.44.1	Test Purpose and Environment	2891

A.9.1.44.2	Test parameters	2891

A.9.1.44.3	Test Requirements	2896

A.9.1.45	4 DL CA PCell in TDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	2896

A.9.1.45.1	Test Purpose and Environment	2896

A.9.1.45.2	Test parameters	2896

A.9.1.45.3	Test Requirements	2416

A.9.1.46	4 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2416

A.9.1.46.1	Test Purpose and Environment	2416

A.9.1.46.2	Test parameters	2416

A.9.1.46.3	Test Requirements	2425

A.9.1.47	4 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2426

A.9.1.47.1	Test Purpose and Environment	2426

A.9.1.47.2	Test parameters	2426

A.9.1.47.3	Test Requirements	2435

A.9.1.48	5 DL FDD-TDD with PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	2435

A.9.1.48.1	Test Purpose and Environment	2435

A.9.1.48.2	Test parameters	2436

A.9.1.48.3	Test Requirements	2446

A.9.1.49	5 DL FDD-TDD with PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	2447

A.9.1.49.1	Test Purpose and Environment	2447

A.9.1.49.2	Test parameters	2447

A.9.1.49.3	Test Requirements	2457

A.9.1.50	5 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2458

A.9.1.50.1	Test Purpose and Environment	2458

A.9.1.50.2	Test parameters	2458

A.9.1.50.3	Test Requirements	2465

A.9.1.51	5 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2466

A.9.1.51.1	Test Purpose and Environment	2466

A.9.1.51.2	Test parameters	2466

A.9.1.51.3	Test Requirements	2472

A.9.1.52	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2473

A.9.1.52.1	Test Purpose and Environment	2473

A.9.1.52.2	Test parameters	2473

A.9.1.52.3	Test Requirements	2476

A.9.1.52A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2476

A.9.1.52A.1	Test Purpose and Environment	2476

A.9.1.52A.2	Test parameters	2476

A.9.1.52A.3	Test Requirements	2477

A.9.1.53	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2477

A.9.1.53.1	Test Purpose and Environment	2477

A.9.1.53.2	Test parameters	2478

A.9.1.53.3	Test Requirements	2481

A.9.1.53A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2481

A.9.1.53A.1	Test Purpose and Environment	2481

A.9.1.53A.2	Test parameters	2481

A.9.1.53A.3	Test Requirements	2482

A.9.1.54	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2482

A.9.1.54.1	Test Purpose and Environment	2482

A.9.1.54.2	Test parameters	2483

A.9.1.54.3	Test Requirements	2485

A.9.1.55	FS3 Intra frequency absolute and relative RSRP accuracies with FDD PCell	2485

A.9.1.55.1	Test Purpose and Environment	2485

A.9.1.55.2	Test parameters	2485

A.9.1.55.3	Test Requirements	2489

A.9.1.56	FS3 Intra frequency absolute and relative RSRP accuracies with TDD PCell	2489

A.9.1.56.1	Test Purpose and Environment	2489

A.9.1.56.2	Test parameters	2489

A.9.1.56.3	Test Requirements	2492

A.9.1.57	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2493

A.9.1.57.1	Test Purpose and Environment	2493

A.9.1.57.2	Test parameters	2493

A.9.1.57.3	Test Requirements	2496

A.9.1.57A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2496

A.9.1.57A.1	Test Purpose and Environment	2496

A.9.1.57A.2	Test parameters	2496

A.9.1.57A.3	Test Requirements	2497

A.9.1.58	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2497

A.9.1.58.1	Test Purpose and Environment	2497

A.9.1.58.2	Test parameters	2498

A.9.1.58.3	Test Requirements	2501

A.9.1.58A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2501

A.9.1.58A.1	Test Purpose and Environment	2501

A.9.1.58A.2	Test parameters	2501

A.9.1.58A.3	Test Requirements	2502

A.9.1.59	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2502

A.9.1.59.1	Test Purpose and Environment	2502

A.9.1.59.2	Test parameters	2503

A.9.1.59.3	Test Requirements	2505

A.9.1.60	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with FDD PCell	2505

A.9.1.60.1	Test Purpose and Environment	2505

A.9.1.60.2	Test parameters	2505

A.9.1.60.3	Test Requirements	2508

A.9.1.61	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with TDD PCell	2509

A.9.1.61.1	Test Purpose and Environment	2509

A.9.1.61.2	Test parameters	2509

A.9.1.61.3	Test Requirements	2512

A.9.1.62	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2512

A.9.1.62.1	Test Purpose and Environment	2512

A.9.1.62.2	Test parameters	2512

A.9.1.62.3	Test Requirements	2515

A.9.1.63	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2515

A.9.1.53.1	Test Purpose and Environment	2515

A.9.1.53.2	Test parameters	2515

A.9.1.63.3	Test Requirements	2518

A.9.1.64	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2518

A.9.1.64.1	Test Purpose and Environment	2518

A.9.1.64.2	Test parameters	2518

A.9.1.64.3	Test Requirements	2519

A.9.1.65	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2520

A.9.1.65.1	Test Purpose and Environment	2520

A.9.1.65.2	Test parameters	2520

A.9.1.65.3	Test Requirements	2526

A.9.1.66	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2526

A.9.1.66.1	Test Purpose and Environment	2526

A.9.1.66.2	Test parameters	2526

A.9.1.66.3	Test Requirements	2532

A.9.1.67	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2532

A.9.1.67.1	Test Purpose and Environment	2532

A.9.1.67.2	Test parameters	2532

A.9.1.67.3	Test Requirements	2533

A.9.1.68	3 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2534

A.9.1.68.1	Test Purpose and Environment	2534

A.9.1.68.2	Test parameters	2534

A.9.1.68.3	Test Requirements	2538

A.9.1.69	4 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2538

A.9.1.69.1	Test Purpose and Environment	2538

A.9.1.69.2	Test parameters	2538

A.9.1.69.3	Test Requirements	2542

A.9.1.70	5 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2543

A.9.1.70.1	Test Purpose and Environment	2543

A.9.1.70.2	Test parameters	2543

A.9.1.70.3	Test Requirements	2555

A.9.1.71	6 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2556

A.9.1.71.1	Test Purpose and Environment	2556

A.9.1.71.2	Test parameters	2556

A.9.1.71.3	Test Requirements	2568

A.9.1.72	7 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2569

A.9.1.72.1	Test Purpose and Environment	2569

A.9.1.72.2	Test parameters	2569

A.9.1.72.3	Test Requirements	2581

A.9.2	RSRQ	2582

A.9.2.1	FDD Intra frequency case	2582

A.9.2.1.1	Test Purpose and Environment	2582

A.9.2.1.2	Test parameters	2582

A.9.2.1.3	Test Requirements	2585

A.9.2.2	TDD Intra frequency case	2585

A.9.2.2.1	Test Purpose and Environment	2585

A.9.2.2.2	Test parameters	2585

A.9.2.2.3	Test Requirements	2587

A.9.2.3	FDD—FDD Inter frequency case	2587

A.9.2.3.1	Test Purpose and Environment	2587

A.9.2.3.2	Test parameters	2587

A.9.2.3.3	Test Requirements	2590

A.9.2.4	TDD—TDD Inter frequency case	2590

A.9.2.4.1	Test Purpose and Environment	2590

A.9.2.4.2	Test parameters	2590

A.9.2.4.3	Test Requirements	2592

A.9.2.4A	FDD—TDD Inter frequency case	2593

A.9.2.4A.1	Test Purpose and Environment	2593

A.9.2.4A.2	Test parameters	2593

A.9.2.4A.3	Test Requirements	2595

A.9.2.5	FDD RSRQ for E-UTRA Carrier Aggregation	2595

A.9.2.5.1	Test Purpose and Environment	2595

A.9.2.5.2	Test parameters	2596

A.9.2.5.3	Test Requirements	2599

A.9.2.6	TDD RSRQ for E-UTRA Carrier Aggregation	2599

A.9.2.6.1	Test Purpose and Environment	2599

A.9.2.6.2	Test parameters	2599

A.9.2.6.3	Test Requirements	2602

A.9.2.7	FDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2602

A.9.2.7.1	Test Purpose and Environment	2602

A.9.2.7.2	Test parameters	2602

A.9.2.7.3	Test Requirements	2606

A.9.2.8	TDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2606

A.9.2.8.1	Test Purpose and Environment	2606

A.9.2.8.2	Test parameters	2606

A.9.2.8.3	Test Requirements	2609

A.9.2.9	FDD RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS	2609

A.9.2.9.1	Test Purpose and Environment	2609

A.9.2.9.2	Test parameters	2609

A.9.2.9.3	Test Requirements	2613

A.9.2.10	TDD Intra frequency case under time domain measurement resource restriction with MBSFN ABS	2613

A.9.2.10.1	Test Purpose and Environment	2613

A.9.2.10.2	Test parameters	2613

A.9.2.10.3	Test Requirements	2617

A.9.2.11	FDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2617

A.9.2.11.1	Test Purpose and Environment	2617

A.9.2.11.2	Test parameters	2617

A.9.2.11.3	Test Requirements	2618

A.9.2.12	TDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2618

A.9.2.12.1	Test Purpose and Environment	2618

A.9.2.12.2	Test parameters	2619

A.9.2.12.3	Test Requirements	2619

A.9.2.13	Void	2620

A.9.2.13.1	Void	2620

A.9.2.13.2	Void	2620

A.9.2.13.3	Void	2620

A.9.2.14	Void	2620

A.9.2.14.1	Void	2620

A.9.2.14.2	Void	2620

A.9.2.14.3	Void	2620

A.9.2.15	FDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2620

A.9.2.15.1	Test Purpose and Environment	2620

A.9.2.15.2	Test parameters	2620

A.9.2.15.3	Test Requirements	2624

A.9.2.16	TDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2624

A.9.2.16.1	Test Purpose and Environment	2624

A.9.2.16.2	Test parameters	2624

A.9.2.16.3	Test Requirements	2629

A.9.2.17	FDD Intra frequency case for 5 MHz bandwidth	2629

A.9.2.17.1	Test Purpose and Environment	2629

A.9.2.17.2	Test parameters	2629

A.9.2.17.3	Test Requirements	2631

A.9.2.18	FDD—FDD Inter frequency case for 5MHz bandwidth	2631

A.9.2.18.1	Test Purpose and Environment	2631

A.9.2.18.2	Test parameters	2631

A.9.2.18.3	Test Requirements	2634

A.9.2.19	FDD-FDD Inter Frequency WB-RSRQ	2634

A.9.2.19.1	Test Purpose and Environment	2634

A.9.2.19.2	Test parameters	2634

A.9.2.19.3	Test Requirements	2636

A.9.2.20	TDD—TDD Inter Frequency WB-RSRQ	2636

A.9.2.20.1	Test Purpose and Environment	2636

A.9.2.20.2	Test parameters	2636

A.9.2.20.3	Test Requirements	2639

A.9.2.21	FDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2639

A.9.2.21.1	Test Purpose and Environment	2639

A.9.2.21.2	Test parameters	2639

A.9.2.21.3	Test Requirements	2642

A.9.2.22	TDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2642

A.9.2.22.1	Test Purpose and Environment	2643

A.9.2.22.2	Test parameters	2643

A.9.2.22.3	Test Requirements	2643

A.9.2.23	FDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2643

A.9.2.23.1	Test Purpose and Environment	2643

A.9.2.23.2	Test parameters	2643

A.9.2.23.3	Test Requirements	2646

A.9.2.24	TDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2646

A.9.2.24.1	Test Purpose and Environment	2646

A.9.2.24.2	Test parameters	2646

A.9.2.24.3	Test Requirements	2647

A.9.2.25	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	2647

A.9.2.25.1	Test Purpose and Environment	2647

A.9.2.25.2	Test parameters	2647

A.9.2.25.3	Test Requirements	2652

A.9.2.26	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	2652

A.9.2.26.1	Test Purpose and Environment	2652

A.9.2.26.2	Test parameters	2652

A.9.2.26.3	Test Requirements	2656

A.9.2.27	TDD RSRQ for E-UTRAN Carrier Aggregation for 20MHz+10MHz	2656

A.9.2.27.1	Test Purpose and Environment	2656

A.9.2.27.2	Test parameters	2656

A.9.2.27.3	Test Requirements	2657

A.9.2.28	FDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2657

A.9.2.28.1	Test Purpose and Environment	2657

A.9.2.28.2	Test parameters	2657

A.9.2.28.3	Test Requirements	2660

A.9.2.29	TDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2660

A.9.2.29.1	Test Purpose and Environment	2660

A.9.2.29.2	Test parameters	2660

A.9.2.29.3	Test Requirements	2663

A.9.2.30	FDD-FDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2663

A.9.2.30.1	Test Purpose and Environment	2663

A.9.2.30.2	Test parameters	2663

A.9.2.30.3	Test Requirements	2666

A.9.2.31	TDD-TDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2666

A.9.2.31.1	Test Purpose and Environment	2666

A.9.2.31.2	Test parameters	2666

A.9.2.31.3	Test Requirements	2669

A.9.2.32	FDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2669

A.9.2.32.1	Test Purpose and Environment	2669

A.9.2.32.2	Test parameters	2669

A.9.2.32.3	Test Requirements	2672

A.9.2.33	TDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2672

A.9.2.33.1	Test Purpose and Environment	2672

A.9.2.33.2	Test parameters	2672

A.9.2.33.3	Test Requirements	2676

A.9.2.34	FDD—FDD Inter frequency new RSRQ	2676

A.9.2.34.1	Test Purpose and Environment	2676

A.9.2.34.2	Test parameters	2676

A.9.2.34.3	Test Requirements	2679

A.9.2.35	TDD—TDD Inter frequency new RSRQ	2679

A.9.2.35.1	Test Purpose and Environment	2679

A.9.2.35.2	Test parameters	2679

A.9.2.35.3	Test Requirements	2682

A.9.2.36	FDD—FDD Inter frequency RSRQ measured on all OFDM symbols	2682

A.9.2.36.1	Test Purpose and Environment	2682

A.9.2.36.2	Test parameters	2682

A.9.2.36.3	Test Requirements	2683

A.9.2.37	TDD—TDD Inter frequency RSRQ measurement on all OFDM symbols	2684

A.9.2.37.1	Test Purpose and Environment	2684

A.9.2.37.2	Test parameters	2684

A.9.2.37.3	Test Requirements	2685

A.9.2.38	3 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2686

A.9.2.38.1	Test Purpose and Environment	2686

A.9.2.38.2	Test parameters	2686

A.9.2.38.3	Test Requirements	2690

A.9.2.39	3 DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2690

A.9.2.39.1	Test Purpose and Environment	2690

A.9.2.39.2	Test parameters	2690

A.9.2.39.3	Test Requirements	2694

A.9.2.40	3 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2694

A.9.2.40.1	Test Purpose and Environment	2694

A.9.2.40.2	Test parameters	2694

A.9.2.40.3	Test Requirements	2697

A.9.2.41	3 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2698

A.9.2.41.1	Test Purpose and Environment	2698

A.9.2.41.2	Test parameters	2698

A.9.2.41.3	Test Requirements	2701

A.9.2.42	FD-FDD RSRQ Intra frequency case for UE category 0	2701

A.9.2.42.1	Test Purpose and Environment	2701

A.9.2.42.2	Test parameters	2701

A.9.2.42.3	Test Requirements	2704

A.9.2.43	HD-FDD RSRQ Intra frequency case for UE category 0	2704

A.9.2.43.1	Test Purpose and Environment	2704

A.9.2.43.2	Test parameters	2704

A.9.2.43.3	Test Requirements	2707

A.9.2.44	TDD RSRQ Intra frequency case for UE category 0	2707

A.9.2.44.1	Test Purpose and Environment	2707

A.9.2.44.2	Test parameters	2707

A.9.2.44.3	Test Requirements	2709

A.9.2.45	4 DL CA PCell in FDD FDD-TDD RSRQ for E-UTRAN in Carrier Aggregation	2709

A.9.2.45.1	Test Purpose and Environment	2709

A.9.2.45.2	Test parameters	2709

A.9.2.45.3	Test Requirements	2713

A.9.2.46	4 DL CA PCell in TDD TDD-FDD RSRQ for E-UTRAN in Carrier Aggregation	2714

A.9.2.46.1	Test Purpose and Environment	2714

A.9.2.46.2	Test parameters	2714

A.9.2.46.3	Test Requirements	2718

A.9.2.47	5 DL FDD-TDD with PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2719

A.9.2.47.1	Test Purpose and Environment	2719

A.9.2.47.2	Test parameters	2719

A.9.2.47.3	Test Requirements	2723

A.9.2.48	5 DL FDD-TDD with PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2723

A.9.2.48.1	Test Purpose and Environment	2723

A.9.2.48.2	Test parameters	2724

A.9.2.48.3	Test Requirements	2728

A.9.2.49	5 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2729

A.9.2.49.1	Test Purpose and Environment	2729

A.9.2.49.2	Test parameters	2729

A.9.2.49.3	Test Requirements	2735

A.9.2.50	5 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2736

A.9.2.50.1	Test Purpose and Environment	2736

A.9.2.50.2	Test parameters	2736

A.9.2.50.3	Test Requirements	2742

A.9.2.51	FS3 Intra frequency absolute and relative RSRQ accuracies with FDD PCell	2742

A.9.2.51.1	Test Purpose and Environment	2742

A.9.2.51.2	Test parameters	2743

A.9.2.51.3	Test Requirements	2747

A.9.2.52	FS3 Intra frequency absolute and relative RSRQ accuracies with TDD PCell	2747

A.9.2.52.1	Test Purpose and Environment	2747

A.9.2.52.2	Test parameters	2747

A.9.2.52.3	Test Requirements	2750

A.9.2.53	4DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2751

A.9.2.53.1	Test Purpose and Environment	2751

A.9.2.53.2	Test parameters	2751

A.9.2.53.3	Test Requirements	2757

A.9.2.54	4DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2758

A.9.2.54.1	Test Purpose and Environment	2758

A.9.2.54.2	Test parameters	2758

A.9.2.54.3	Test Requirements	2764

A.9.2.55	3 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2764

A.9.2.55.1	Test Purpose and Environment	2764

A.9.2.55.2	Test parameters	2765

A.9.2.55.3	Test Requirements	2770

A.9.2.56	4 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2770

A.9.2.56.1	Test Purpose and Environment	2770

A.9.2.56.2	Test parameters	2770

A.9.2.56.3	Test Requirements	2775

A.9.2.57	5 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2775

A.9.2.57.1	Test Purpose and Environment	2775

A.9.2.57.2	Test parameters	2776

A.9.2.57.3	Test Requirements	2780

A.9.2.58	6 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2781

A.9.2.58.1	Test Purpose and Environment	2781

A.9.2.58.2	Test parameters	2781

A.9.2.58.3	Test Requirements	2785

A.9.2.59	7 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2786

A.9.2.59.1	Test Purpose and Environment	2786

A.9.2.59.2	Test parameters	2786

A.9.2.59.3	Test Requirements	2791

A.9.3	UTRAN FDD CPICH RSCP	2792

A.9.3.1	E-UTRAN FDD	2792

A.9.3.1.1	Test Purpose and Environment	2792

A.9.3.1.2	Parameters	2792

A.9.3.1.3	Test Requirements	2794

A.9.3.2 E-UTRAN TDD	2794

A.9.3.2.1	Test Purpose and Environment	2794

A.9.3.2.2	Parameters	2794

A.9.3.2.3	Test Requirements	2797

A.9.3.3	E-UTRAN FDD for 5MHz Bandwidth	2797

A.9.3.3.1	Test Purpose and Environment	2797

A.9.3.3.2	Parameters	2797

A.9.3.3.3	Test Requirements	2798

A.9.4	UTRAN FDD CPICH Ec/No	2799

A.9.4.1	E-UTRAN FDD	2799

A.9.4.1.1	Test Purpose and Environment	2799

A.9.4.1.2	Parameters	2799

A.9.4.1.3	Test Requirements	2801

A.9.4.2	E-UTRAN TDD	2802

A.9.4.2.1	Test Purpose and Environment	2802

A.9.4.2.2	Parameters	2802

A.9.4.2.3	Test Requirements	2804

A.9.4.3	E-UTRAN FDD for 5MHz Bandwidth	2805

A.9.4.3.1	Test Purpose and Environment	2805

A.9.4.3.2	Parameters	2805

A.9.4.3.3	Test Requirements	2806

A.9.5	UTRAN TDD measurement	2806

A.9.5.1	P-CCPCH RSCP absolute accuracy for E-UTRAN FDD	2806

A.9.5.1.1	Test Purpose and Environment	2806

A.9.5.1.2	Test parameters	2806

A.9.5.1.3	Test Requirements	2808

A.9.5.2	P-CCPCH RSCP absolute accuracy for E-UTRAN TDD	2808

A.9.5.2.1	Test Purpose and Environment	2808

A.9.5.2.2	Test parameters	2808

A.9.5.2.3	Test Requirements	2810

A.9.6	GSM Carrier RSSI	2811

A.9.6.1	E-UTRAN FDD	2811

A.9.6.1.1	Test Purpose and Environment	2811

A.9.6.1.2	Test Requirements	2812

A.9.6.2	E-UTRAN TDD	2812

A.9.6.2.1	Test Purpose and Environment	2812

A.9.6.2.2	Test Requirements	2814

A.9.7	UE Rx – Tx Time Difference	2815

A.9.7.1	E-UTRAN FDD UE Rx – Tx time difference case	2815

A.9.7.1.1	Test Purpose and Environment	2815

A.9.7.1.2	Test parameters	2815

A.9.7.1.3	Test Requirements	2816

A.9.7.2	E-UTRA TDD	2816

A.9.7.2.1	Test Purpose and Environment	2816

A.9.7.2.2	Test parameters	2816

A.9.7.2.3	Test Requirements	2818

A.9.7.3	E-UTRAN FDD UE Rx–Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2818

A.9.7.3.1	Test Purpose and Environment	2818

A.9.7.3.2	Test parameters	2818

A.9.7.3.3	Test Requirements	2821

A.9.7.4	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2821

A.9.7.4.1	Test Purpose and Environment	2821

A.9.7.4.2	Test Parameters	2821

A.9.7.4.3	Test Requirements	2824

A.9.7.5	E-UTRAN FDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2824

A.9.7.5.1	Test Purpose and Environment	2824

A.9.7.5.2	Test parameters	2824

A.9.7.5.3	Test Requirements	2827

A.9.7.6	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2827

A.9.7.6.1	Test Purpose and Environment	2827

A.9.7.6.2	Test Parameters	2827

A.9.7.6.3	Test Requirements	2830

A.9.7.7	E-UTRAN FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2830

A.9.7.7.1	Test Purpose and Environment	2830

A.9.7.7.2	Test parameters	2830

A.9.7.7.3	Test Requirements	2831

A.9.7.8	E-UTRAN HD-FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2832

A.9.7.8.1	Test Purpose and Environment	2832

A.9.7.8.2	Test parameters	2832

A.9.7.8.3	Test Requirements	2833

A.9.7.9	E-UTRAN TDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2833

A.9.7.9.1	Test Purpose and Environment	2833

A.9.7.9.2	Test parameters	2833

A.9.7.9.3	Test Requirements	2834

A.9.8	RSTD	2835

A.9.8.1	E-UTRAN FDD RSTD intra frequency case	2835

A.9.8.1.1	Test Purpose and Environment	2835

A.9.8.1.2	Test Requirements	2837

A.9.8.1.2A	Test Requirements for UE Category 1bis	2837

A.9.8.2	E-UTRAN TDD RSTD intra frequency case	2838

A.9.8.2.1	Test Purpose and Environment	2838

A.9.8.2.2	Test Requirements	2841

A.9.8.2.2A	Test Requirements for UE Category 1bis	2841

A.9.8.3	E-UTRAN FDD-FDD RSTD inter frequency case	2842

A.9.8.3.1	Test Purpose and Environment	2842

A.9.8.3.2	Test Requirements	2844

A.9.8.3.2A	Test Requirements for UE Category 1bis	2844

A.9.8.4	E-UTRAN TDD-TDD RSTD inter frequency case	2845

A.9.8.4.1	Test Purpose and Environment	2845

A.9.8.4.2	Test Requirements	2847

A.9.8.4.2A	Test Requirements for UE Category 1bis	2847

A.9.8.5	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation	2848

A.9.8.5.1	Test Purpose and Environment	2848

A.9.8.5.2	Test Requirements	2850

A.9.8.6	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation	2850

A.9.8.6.1	Test Purpose and Environment	2850

A.9.8.6.2	Test Requirements	2853

A.9.8.7	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2853

A.9.8.7.1	Test Purpose and Environment	2853

A.9.8.7.2	Test Requirements	2854

A.9.8.8	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2854

A.9.8.8.1	Test Purpose and Environment	2854

A.9.8.8.2	Test Requirements	2855

A.9.8.9	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2855

A.9.8.9.1	Test Purpose and Environment	2855

A.9.8.9.2	Test Requirements	2856

A.9.8.10	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2856

A.9.8.10.1	Test Purpose and Environment	2856

A.9.8.10.2	Test Requirements	2857

A.9.8.11	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 5 + 5MHz bandwidth	2857

A.9.8.11.1	Test Purpose and Environment	2857

A.9.8.11.2	Test Requirements	2858

A.9.8.12	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 5+5MHz bandwidth	2858

A.9.8.12.1	Test Purpose and Environment	2858

A.9.8.12.2	Test Requirements	2859

A.9.8.13	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz+10MHz	2859

A.9.8.13.1	Test Purpose and Environment	2859

A.9.8.13.2	Test Requirements	2860

A.9.8.14	E-UTRAN FDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2860

A.9.8.14.1	Test Purpose and Environment	2860

A.9.8.14.2	Test Requirements	2867

A.9.8.15	E-UTRAN TDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2867

A.9.8.15.1	Test Purpose and Environment	2867

A.9.8.15.2	Test Requirements	2873

A.9.8.16	HD – FDD Intra frequency case for UE Category NB1 inband mode in normal coverage	2873

A.9.8.16.1	Test Purpose and Environment	2873

A.9.8.16.2	Test Requirements	2876

A.9.8.17	HD – FDD Inter frequency case for UE Category NB1 inband mode in normal coverage	2876

A.9.8.17.1	Test Purpose and Environment	2876

A.9.8.17.2	Test Requirements	2879

A.9.8.18	HD – FDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	2879

A.9.8.18.1	Test Purpose and Environment	2879

A.9.8.18.2	Test Requirements	2882

A.9.8.19	HD – FDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	2882

A.9.8.19.1	Test Purpose and Environment	2882

A.9.8.19.2	Test Requirements	2885

A.9.8.20	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode A	2885

A.9.8.20.1	Test Purpose and Environment	2885

A.9.8.20.2	Test Requirements	2888

A.9.8.21	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CEModeA	2889

A.9.8.21.1	Test Purpose and Environment	2889

A.9.8.21.2	Test Requirements	2892

A.9.8.22	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode A	2893

A.9.8.22.1	Test Purpose and Environment	2893

A.9.8.22.2	Test Requirements	2896

A.9.8.23	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode B	2897

A.9.8.23.1	Test Purpose and Environment	2897

A.9.8.23.2	Test Requirements	2900

A.9.8.24	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CE Mode B	2900

A.9.8.24.1	Test Purpose and Environment	2900

A.9.8.24.2	Test Requirements	2904

A.9.8.25	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode B	2905

A.9.8.25.1	Test Purpose and Environment	2905

A.9.8.25.2	Test Requirements	2908

A.9.8.26	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	2909

A.9.8.26.1	Test Purpose and Environment	2909

A.9.8.26.2	Test Requirements	2911

A.9.8.27	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	2911

A.9.8.27.1	Test Purpose and Environment	2911

A.9.8.27.2	Test Requirements	2914

A.9.8.28	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode A	2914

A.9.8.28.1	Test Purpose and Environment	2914

A.9.8.28.2	Test Requirements	2917

A.9.8.29	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	2917

A.9.8.29.1	Test Purpose and Environment	2917

A.9.8.29.2	Test Requirements	2921

A.9.8.30	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	2922

A.9.8.30.1	Test Purpose and Environment	2922

A.9.8.30.2	Test Requirements	2925

A.9.8.31	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode B	2926

A.9.8.31.1	Test Purpose and Environment	2926

A.9.8.31.2	Test Requirements	2930

A.9.8.32	TDD Intra frequency case for UE Category NB1 inband mode in normal coverage	2930

A.9.8.32.1	Test Purpose and Environment	2930

A.9.8.32.2	Test Requirements	2933

A.9.8.33	TDD Inter frequency case for UE Category NB1 inband mode in normal coverage	2933

A.9.8.33.1	Test Purpose and Environment	2933

A.9.8.33.2	Test Requirements	2936

A.9.8.34	TDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	2936

A.9.8.34.1	Test Purpose and Environment	2936

A.9.8.34.2	Test Requirements	2939

A.9.8.35	TDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	2939

A.9.8.35.1	Test Purpose and Environment	2939

A.9.8.35.2	Test Requirements	2942

A.9.9	RSRP and RSRQ on the serving cell	2942

A.9.9.1	FDD Intra frequency serving cell case	2942

A.9.9.1.1	Test Purpose and Environment	2942

A.9.9.1.2	Test parameters	2942

A.9.9.1.3	Test Requirements	2945

A.9.9.2	TDD Intra frequency serving cell case	2945

A.9.9.2.1	Test Purpose and Environment	2945

A.9.9.2.2	Test parameters	2945

A.9.9.2.3	Test Requirements	2947

A.9.10	SSTD	2947

A.9.10.1	EUTRAN FDD-FDD SSTD accuracy in asynchronous DC	2947

A.9.10.1.1	Test Purpose and Environment	2947

A.9.10.1.2	Test parameters	2947

A.9.10.1.3	Test Requirements	2949

A.9.10.2	Void	2949

A.9.10.3	Void	2949

A.9.10.4	Void	2949

A.9.11 RSSI	2949

A.9.11.1	FS3 average RSSI accuracy case (PCell using FDD)	2949

A.9.11.1.1	Test Purpose and Environment	2949

A.9.11.1.2	Test parameters	2949

A.9.11.1.3	Test Requirements	2952

A.9.11.2	FS3 average RSSI accuracy case (PCell using TDD)	2952

A.9.11.2.1	Test Purpose and Environment	2952

A.9.11.2.2	Test parameters	2952

A.9.12	Channel occupancy	2955

A.9.12.1	FS3 channel occupancy test (PCell using FDD)	2955

A.9.12.1.1	Test Purpose and Environment	2955

A.9.12.1.2	Test parameters	2955

A.9.12.1.3	Test Requirements	2958

A.9.12.2	FS3 channel occupancy test (PCell using TDD)	2958

A.9.12.2.1	Test Purpose and Environment	2958

A.9.12.2.2	Test parameters	2958

A.9.12.2.3	Test Requirements	2961

A.9.13	RS-SINR	2961

A.9.13.1	FDD Intra-Frequency Case	2961

A.9.13.1.1	Test Purpose and Environment	2961

A.9.13.1.2	Test parameters	2961

A.9.13.1.3	Test Requirements	2964

A.9.13.2	TDD Intra-Frequency Case	2964

A.9.13.2.1	Test Purpose and Environment	2964

A.9.13.2.2	Test parameters	2964

A.9.13.2.3	Test Requirements	2967

A.9.13.3	FDD—FDD Inter frequency case	2967

A.9.13.3.1	Test Purpose and Environment	2967

A.9.13.3.2	Test parameters	2967

A.9.13.3.3	Test Requirements	2970

A.9.13.4	TDD—TDD Inter frequency case	2970

A.9.13.4.1	Test Purpose and Environment	2970

A.9.13.4.2	Test parameters	2970

A.9.13.4.3	Test Requirements	2974

A.9.13.5	FDD—TDD Inter frequency case	2975

A.9.13.5.1	Test Purpose and Environment	2975

A.9.13.5.2	Test parameters	2975

A.9.13.5.3	Test Requirements	2981

A.9.13.6	TDD—FDD Inter frequency case	2981

A.9.13.6.1	Test Purpose and Environment	2981

A.9.13.6.2	Test parameters	2981

A.9.13.6.3	Test Requirements	2985

A.9.14	Channel quality reporting accuracy	2985

A.9.14.1	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage	2985

A.9.14.1.1	Test Purpose and Environment	2985

A.9.14.1.2	Test parameters	2986

A.9.14.1.3	Test Requirements	2986

A.9.14.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage	2987

A.9.14.2.1	Test Purpose and Environment	2987

A.9.14.2.2	Test parameters	2987

A.9.14.2.3	Test Requirements	2988

A.10	Proximity-based Services in Any Cell Selection State	2988

A.10.1	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	2988

A.10.1.1	Test Purpose and Environment	2988

A.10.1.2	Test Requirements	2989

A.10.2	E-UTRAN FDD – Initiation/Cease of SLSS Transmission with ProSe Direct Communication	2990

A.10.2.1	Test Purpose and Environment	2990

A.10.2.2	Test Requirements	2991

A.10.3	E-UTRAN FDD – SyncRef UE Selection / Reselection Test	2991

A.10.3.1	Test Purpose and Environment	2991

A.10.3.2	Test Requirements	2993

A.10.4	E-UTRAN FDD – Cell Identification on downlink frequency associated with ProSe frequency (when UE is transmitting for ProSe)	2994

A.10.4.1	Test Purpose and Environment	2994

A.10.4.2	Test Requirements	2996

A.11	V2V Sidelink Communication for V2V Operation on Dedicated V2V Carrier	2996

A.11.1	V2V UE Transmission Timing Accuracy Test	2996

A.11.1.1	Test Purpose and Environment	2996

A.11.1.2	Test requirements	2997

A.11.2	Interruptions due to V2V sidelink communication	2997

A.11.2.1	Test Purpose and Environment	2997

A.11.2.2	Test Requirements	2999

A.12	2999

A.12.1	V2X UE Transmission Timing Accuracy Test	2999

A.12.1.1	V2X UE Transmission Timing Accuracy Test for eNB as Timing Reference	2999

A.12.1.1.1	Test Purpose and Environment	2999

A.12.1.1.2	Test requirements	3000

A.12.1.2	V2X UE Transmission Timing Accuracy Test for SyncRef UE as Timing Reference	3001

A.12.1.2.1	Test Purpose and Environment	3001

A.12.1.2.2	Test Requirements	3001

A.12.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication	3002

A.12.2.1	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for eNB as Timing Reference	3002

A.12.2.1.1	Test Purpose and Environment	3002

A.12.2.1.2	Test Requirements	3003

A.12.2.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for SyncRef UE as Timing Reference	3004

A.12.2.2.1	Test Purpose and Environment	3004

A.12.2.2.2	Test Requirements	3005

A.12.3	V2X Synchronization Reference Selection/Reselection Tests	3005

A.12.3.1	V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority	3005

A.12.3.1.1	Test Purpose and Environment	3005

A.12.3.1.2	Test Requirements	3008

A.12.3.2	V2X Synchronization Reference Selection/Reselection Tests for eNB configured as the highest priority	3009

A.12.3.2.1	Test Purpose and Environment	3009

A.12.3.1.2	Test Requirements	3011

A.12.4	Congestion Control Measurement Test for V2X UE	3011

A.12.4.1	Test Purpose and Environment	3011

A.12.4.2	Test Requirements	3013

A.12.5	Interruptions due to V2X Sidelink Communication	3013

A.12.5.1	Test Purpose and Environment	3013

A.12.5.1	Test Requirements	3015

A.12.6	V2X UE Autonomous Resource Selection/Reselection Measurement Test	3015

A.12.6.1	V2X UE Autonomous Resource Selection/Reselection Tests for PSSCH-RSRP measurements	3015

A.12.6.1.1	Test Purpose and Environment	3015

A.12.6.1.2	Test Requirements	3017

A.12.6.2	V2X UE Autonomous Resource Selection/Reselection Tests for S-RSSI measurements	3017

A.12.6.2.1	Test Purpose and Environment	3017

A.12.6.1.2	Test Requirements	3019

A.12.7	V2X Synchronization Reference Selection/Reselection Tests for V2X Carrier Aggregation	3019

A.12.7.1	Test Purpose and Environment	3019

A.12.7.2	Test Requirements	3022

A.12.8	Interruptions due to V2X Carrier Aggregation	3023

A.12.8.1	Interruptions on a FDD PCell	3023

A.12.8.1.1	Test Purpose and Environment	3023

A.12.8.1.2	Test Requirements	3024

A.12.8.2	Interruptions on a TDD PCell	3024

A.12.8.2.1	Test Purpose and Environment	3024

A.12.8.2.2	Test Requirements	3026

Annex B (normative):	Conditions for RRM requirements applicability for operating bands	3027

B.1	Conditions for E-UTRAN RRC_IDLE state mobility	3027

B.1.1	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection	3027

B.1.2	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection	3027

B.1.3	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category M1	3027

B.1.4	Conditions for measurements of intra-frequency NB-IoT cells for cell re-selection for UE Category NB1	3029

B.1.5	Conditions for measurements of inter-frequency NB-IoT cells for cell re-selection for UE Category NB1	3030

B.1.6	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category 1bis	3030

B.1.7	Conditions for measurements of E-UTRAN cells for cell re-selection for UE Category M2	3030

B.1.7.1	Conditions for measurements of intra-frequence E-UTRAN cells for cell selection	3030

B.1.7.2	Condition for measurements of inter-frequence E-UTRAN cells for cell selection	3032

B.1.8	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection for UE Category M1	3033

B.2	Conditions for UE Measurements Procedures in RRC_CONNECTED State	3033

B.2.1	Conditions for E-UTRAN intra-frequency measurements	3033

B.2.2	Conditions for E-UTRAN intra-frequency measurements with autonomous gaps	3034

B.2.3	Conditions for E-UTRAN inter-frequency measurements	3034

B.2.4	Conditions for E-UTRAN inter-frequency measurements with autonomous gaps	3035

B.2.5	Conditions for E-UTRAN OTDOA intra-frequency RSTD Measurements	3035

B.2.6	Conditions for E-UTRAN OTDOA inter-frequency RSTD Measurements	3035

B.2.7	Conditions for Measurements of the secondary component carrier with deactivated SCell	3036

B.2.8	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction	3036

B.2.9	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3037

B.2.10	Conditions for E-UTRAN intra-frequency discovery signal measurements	3037

B.2.10.1	Conditions for E-UTRAN intra-frequency CRS-based measurements	3037

B.2.10.2	Conditions for E-UTRAN intra-frequency CSI-RS based measurements	3037

B.2.11	Conditions for E-UTRAN inter-frequency discovery signal measurements	3038

B.2.11.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	3038

B.2.11.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	3038

B.2.12	Conditions for E-UTRAN intra-frequency discovery signal measurements under operation with frame structure 3	3039

B.2.13	Conditions for E-UTRAN inter-frequency discovery signal measurements under operation with frame structure 3	3039

B.2.13.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	3039

B.2.13.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	3040

B.2.14	Conditions for E-UTRAN intra-frequency measurements by UE Category M1	3040

B.2.15	Conditions for NB-IoT intra-frequency measurements by UE Category NB1	3041

B.2.16	Conditions for NB-IoT intra-frequency RSTD measurements by UE Category NB1	3042

B.2.17	Conditions for NB-IoT inter-frequency RSTD measurements by UE Category NB1	3043

B.2.18	Conditions for E-UTRAN inter-frequency measurements by UE Category M1	3044

B.2.19	Conditions for E-UTRAN measurements by UE Category M2	3045

B.2.19.1	Conditions for E-UTRAN intra-frequency measurements	3045

B.2.19.2	Conditions for E-UTRAN inter-frequency measurements	3045

B.2.20	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M1	3045

B.2.21	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M2	3046

B.2.22	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M1	3046

B.2.23	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M2	3048

B.3	Conditions for measurements performance requirements for UE	3048

B.3.1	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements	3048

B.3.2	Void	3048

B.3.3	Conditions for inter-frequency RSRP and RSRQ Accuracy Requirements	3048

B.3.4	Conditions for inter-frequency relative RSRP and RSRQ Accuracy Requirements	3048

B.3.5	Conditions for UE Rx – Tx time difference	3048

B.3.6	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements	3049

B.3.7	Conditions for inter-frequency RSTD measurements	3049

B.3.8	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements	3049

B.3.9	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction	3049

B.3.10	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction	3049

B.3.11	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3050

B.3.12	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3050

B.3.13	Conditions for UE Rx–Tx Time Difference Measurement under Time Domain Measurement Resource Restriction with CRS Assistance Information	3050

B.3.14	Conditions for Intra-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	3050

B.3.14.1	Conditions for Intra-frequency CRS-based measurements	3050

B.3.14.2	Conditions for Intra-frequency CSI-RS-based measurements	3050

B.3.15	Conditions for Intra-Frequency Relative Discovery Signal Measurement Accuracy Requirements	3051

B.3.15.1	Conditions for Intra-frequency CRS-based measurements	3051

B.3.15.2	Conditions for Intra-frequency CSI-RS-based measurements	3051

B.3.16	Conditions for Inter-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	3052

B.3.16.1	Conditions for Inter-frequency CRS-based measurements	3052

B.3.16.2	Conditions for Inter-frequency CSI-RS-based measurements	3052

B.3.17	Conditions for Inter-Frequency Relative Discovery Signal Measurement Accuracy Requirements	3052

B.3.17.1	Conditions for Inter-frequency CRS-based measurements	3052

B.3.17.2	Conditions for Inter-frequency CSI-RS-based measurements	3052

B.3.18	Conditions for Intra-frequency Absolute RS-SINR Accuracy Requirements	3052

B.3.19	Conditions for Inter-frequency Absolute RS-SINR Accuracy Requirements	3052

B.3.20	Conditions for Inter-frequency Relative RS-SINR Accuracy Requirements	3052

B.3.21	Conditions for Intra-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	3053

B.3.21.1	Conditions for RSRP measurements	3053

B.3.21.2	Conditions for RSRQ measurements	3053

B.3.21.3	Conditions for CSI-RSRP measurements	3053

B.3.22	Conditions for Intra-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	3053

B.3.22.1	Conditions for RSRP measurements	3053

B.3.22.2	Void	3054

B.3.22.3	Conditions for CSI-RSRP measurements	3054

B.3.23	Conditions for Inter-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	3054

B.3.23.1	Conditions for RSRP measurements	3054

B.3.23.2	Conditions for RSRQ measurements	3054

B.3.23.3	Conditions for CSI-RSRP measurements	3054

B.3.24	Conditions for Inter-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	3055

B.3.24.1	Conditions for RSRP measurements	3055

B.3.24.2	Conditions for RSRQ measurements	3055

B.3.24.3	Conditions for CSI-RSRP measurements	3055

B.3.25	Conditions for NB-IoT intra-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	3055

B.3.26	Conditions for NB-IoT inter-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	3055

B.3.27	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements for Category 0	3055

B.3.28	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements for Category 0	3056

B.3.29	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for NB1	3056

B.3.30	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for NB1	3056

B.3.31	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	3056

B.3.32	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	3056

B.3.33	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	3057

B.3.34	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	3057

B.4	RRM Requirements Exceptions	3057

B.4.1	General	3057

B.4.2	Receiver sensitivity relaxation for UE supporting CA	3057

B.4.3	Receiver sensitivity relaxation for UE configured with CA	3057

B.4.3.1	Inter-band carrier aggregation	3057

B.4.3.2	Intra-band non-contiguous carrier aggregation	3057

B.4.3.3	Inter-band carrier aggregation with operating bands without uplink band	3058

B.5	Conditions for Measurement Performance Requirements for ProSe UE	3058

B.5.1	Conditions for S-RSRP Accuracy Requirements	3058

B.5.2	Conditions for Relative S-RSRP Accuracy Requirements	3058

B.5.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	3059

B.5.4	Conditions for SD-RSRP Accuracy Requirements	3059

B.5.5	Conditions for Relative SD-RSRP Accuracy Requirements	3060

B.6	Conditions for V2X	3060

B.6.1	Test parameters for GNSS signals	3060

B.6.2	Conditions for Absolute S-RSRP Accuracy Requirements	3060

B.6.3	Conditions for Relative S-RSRP Accuracy Requirements	3060

B.6.4	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	3061

B.6.5	Conditions for Absolute PSSCH-RSRP Accuracy Requirements	3061

B.7	Conditions for sTTI and 1ms-TTI with 3 Subframe HARQ Processing	3061

B.7.1	Conditions for Maximum Timing Difference Between Uplink and Downlink Carriers in Carrier Aggregation	3061

Annex C (informative):	Change history:	3063