| 3GPP TS 36.133 V16.8.0 (2020-12) |  |
| --- | --- |
| Technical Specification¦Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	92

1	Scope	94

2	References	94

3	Definitions, symbols and abbreviations	96

3.1	Definitions	96

3.2	Symbols	98

3.3	Abbreviations	99

3.4	Test tolerances	102

3.5	Additional notation	102

3.5.1	Groups of bands	102

3.6	General	104

3.6.1	Applicability of requirements in this specification version	104

3.6.1.1	Applicability of requirements for UE capable of network-based CRS interference mitigation	110

3.6.1.2	Applicability of requirements with CRS muting for category M1 UE capable of CRS muting	112

3.6.1.3	Applicability of requirements with CRS muting for category M2 UE capable of CRS muting	113

3.6.2	Applicability of requirements for EN-DC operation	114

3.6.3	Applicability of requirements for NE-DC operation	115

3.6.4	Applicability of requirements for NGEN-DC operation	116

4	E-UTRAN RRC_IDLE state mobility	116

4.1	Cell Selection	116

4.2	Cell Re-selection	116

4.2.1	Introduction	116

4.2.2	Requirements	117

4.2.2.1	Measurement and evaluation of serving cell	118

4.2.2.2	Void	119

4.2.2.3	Measurements of intra-frequency E-UTRAN cells	119

4.2.2.4	Measurements of inter-frequency E-UTRAN cells	120

4.2.2.5	Measurements of inter-RAT cells	123

4.2.2.5.1	Measurements of UTRAN FDD cells	123

4.2.2.5.2	Measurements of UTRAN TDD cells	124

4.2.2.5.3	Measurements of GSM cells	126

4.2.2.5.4	Measurements of HRPD cells	127

4.2.2.5.5	Measurements of cdma2000 1X	128

4.2.2.5.6	Measurements of NR cells	129

4.2.2.5.7	Measurements of NR cells subject to CCA	131

4.2.2.6	Evaluation of cell re-selection criteria	132

4.2.2.7	Maximum interruption in paging reception	133

4.2.2.8	void	133

4.2.2.9	UE measurement capability	133

4.2.2.9a	UE measurement capability (Increased UE carrier monitoring)	134

4.2.2.10	Reselection to CSG cells	134

4.2.2.10.1	Reselection from a non CSG to an inter-frequency CSG cell	135

4.2.2.10.2	Reselection from a non CSG to an inter-RAT UTRAN FDD CSG cell	135

4.2.2.11	Void	136

4.2.2.12	Void	136

4.2.2.13	Void	136

4.3	Minimization of Drive Tests (MDT)	136

4.3.1	Introduction	137

4.3.2	Measurements	137

4.3.2.1	Requirements	137

4.3.3	Relative Time Stamp Accuracy	137

4.3.3.1	Requirements	137

4.3.4	Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	137

4.3.4.1	 Requirements	137

4.3.5	Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	138

4.3.5.1	Requirements for timeSinceFailure	138

4.4	MBSFN Measurements	138

4.4.1	Introduction	138

4.4.2	MBSFN RSRP measurements	138

4.4.3	MBSFN RSRQ measurements	138

4.4.4	MCH BLER measurements	139

4.5	Proximity-based Services	139

4.5.1	Introduction	139

4.5.2	Requirements	139

4.5.2.1	Interruptions with ProSe Direct Discovery	139

4.5.2.2	Interruptions with ProSe Direct Communication	139

4.5.2.3	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	139

4.5.2.4	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	140

4.6	Cell Selection and Re-selection Requirements for UE category NB1	140

4.6.1	Cell Selection	140

4.6.2	Cell Re-selection	140

4.6.2.1	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage	141

4.6.2.1A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in normal coverage when configured with WUS	142

4.6.2.2	Measurements of intra-frequency NB-IoT cells for UE category NB1 in normal coverage	143

4.6.2.3	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in enhanced coverage	144

4.6.2.3A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in enhanced coverage when configured with WUS	145

4.6.2.4	Measurements of intra-frequency NB-IoT cells for UE category NB1 in enhanced coverage	146

4.6.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage	148

4.6.2.6	Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage	149

4.6.2.7	Maximum interruption in paging reception in normal coverage	150

4.6.2.7A	Maximum interruption in paging reception in enhanced coverage	151

4.6.2.8	UE measurement capability	151

4.6.2.9	WUS receptions for NB1	151

4.6.3	Requirements for transmission using preconfigured uplink resources for UE category NB1	151

4.6.3.1	Introduction	151

4.6.3.2	Requirements on UE synchronization for transmission using PUR	151

4.6.3.3	Requirements on TA validation for transmission using PUR	152

4.7	Cell Selection and Re-selection Requirements for UE category M1	152

4.7.1	Cell Selection	153

4.7.2	Cell Re-selection	153

4.7.2.1	Cell Re-selection requirements for UE category M1 in normal coverage	153

4.7.2.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	153

4.7.2.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	154

4.7.2.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	156

4.7.2.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	158

4.7.2.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	158

4.7.2.2	Cell Re-selection requirements for UE category M1 in enhanced coverage	159

4.7.2.2.1	Measurement and evaluation of serving cell for UE category M1 in enhanced coverage	159

4.7.2.2.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in enhaned coverage	160

4.7.2.2.2	Measurements of intra-frequency cells for UE category M1 in enhanced coverage	161

4.7.2.2.3	Measurements of inter-frequency cells for UE category M1 in enhanced coverage	163

4.7.2.2.4	Maximum allowed layers for multiple monitoring for UE category M1 in enhanced coverage	165

4.7.2.2.5	Maximum interruption in paging reception for Category M1 UEs in extended coverage	166

4.7.2.3	WUS receptions for UE category M1	166

4.7.3	Channel quality report for UE Category M1 in idle mode	166

4.7.4	Requirements for transmission using preconfigured uplink resources for UE category M1	167

4.7.4.1	Introduction	167

4.7.4.2	Requirements on UE synchronization for transmission using PUR	167

4.7.4.3	Requirements on TA validation for transmission using PUR	167

4.8	Idle State Positioning Measurement Requirements for UE category NB1	168

4.8.1	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for normal coverage	168

4.8.1.1	RSTD Measurement Reporting Delay	170

4.8.2	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	170

4.8.2.1	RSTD Measurement Reporting Delay	171

4.8.3	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for normal coverage	172

4.8.3.1	RSTD Measurement Reporting Delay	173

4.8.4	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	174

4.8.4.1	RSTD Measurement Reporting Delay	175

4.8.5	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	176

4.8.5.1	Measurement Reporting Delay	177

4.8.6	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	177

4.8.6.1	Measurement Reporting Delay	178

4.8.7	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	179

4.8.7.1	Measurement Reporting Delay	180

4.8.8	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	180

4.8.8.1	Measurement Reporting Delay	181

4.9	Idle Mode CA Measurement	182

4.9.1	Introduction	182

4.9.2	Requirement	182

4.9.2.1	Detected cell requirement during state transition and Idle mode	182

4.9.2.2	Measurements of inter-frequency CA candidate cells	183

4.9.2.3	Measurements on serving cell	183

4A	E-UTRAN RRC_INACTIVE state mobility	184

4A.1	Cell Re-selection	184

4A.1.1	Introduction	184

4A.1.2	Requirements	185

4A.1.2.1	UE measurement capability	185

4A.1.2.2	Measurement and evaluation of serving cell	185

4A.1.2.3	Measurements of intra-frequency E-UTRAN cells	185

4A.1.2.4	Measurements of inter-frequency E-UTRAN cells	185

4A.1.2.5	Evaluation of cell re-selection criteria	185

4A.1.2.6	Maximum interruption in paging reception	185

4A.1.2.7	Measurements of inter-RAT NR cells	185

4A.2	Requirements for UE Category M1	185

4A.2.1	Introduction	185

4A.2.2	Cell Selection	185

4A.2.3	Cell Reselection	185

4A.2.3.1	Cell Re-selection requirements for UE category M1 in normal coverage	185

4A.2.3.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	185

4A.2.3.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	185

4A.2.3.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	185

4A.2.3.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	186

4A.2.3.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	186

4A.2.4	Channel quality report for UE Category M1 in idle mode	186

5	E-UTRAN RRC_CONNECTED state mobility	186

5.1	E-UTRAN Handover	187

5.1.1	Introduction	187

5.1.2	Requirements	187

5.1.2.1	E-UTRAN FDD – FDD	187

5.1.2.1.1	Handover delay	187

5.1.2.1.2	Interruption time	187

5.1.2.2	E-UTRAN FDD – TDD	189

5.1.2.2.1	(Void)	189

5.1.2.2.2	(Void)	189

5.1.2.3	E-UTRAN TDD – FDD	189

5.1.2.3.1	(Void)	189

5.1.2.3.2	(Void)	189

5.1.2.4	E-UTRAN TDD – TDD	189

5.1.2.4.1	Handover delay	189

5.1.2.4.2	Interruption time	189

5.1.2.5	E-UTRAN HD–FDD	191

5.1.2.5.1	Handover delay	191

5.1.2.5.2	Interruption time	191

5.1.2.6	E-UTRAN FDD – FDD conditional handover	192

5.1.2.6.1	Handover delay	192

5.1.2.6.2	Measurement time	193

5.1.2.6.3	Preparation time	193

5.1.2.6.4	Interruption time	193

5.1.2.7	E-UTRAN FDD – TDD conditional handover	194

5.1.2.8	E-UTRAN TDD – FDD conditional handover	194

5.1.2.9	E-UTRAN TDD – TDD conditional handover	194

5.2	Void	194

5.3	Handover to other RATs	194

5.3.1	E-UTRAN - UTRAN FDD Handover	194

5.3.1.1	Introduction	194

5.3.1.1.1	Handover delay	194

5.3.1.1.2	Interruption time	194

5.3.2	E-UTRAN - UTRAN TDD Handover	195

5.3.2.1	Introduction	195

5.3.2.2	Requirements	195

5.3.2.2.1	Handover delay	195

5.3.2.2.2	Interruption time	195

5.3.3	E-UTRAN - GSM Handover	196

5.3.3.1	Introduction	196

5.3.3.2	Requirements	196

5.3.3.2.1	Handover delay	196

5.3.3.2.2	Interruption time	196

5.3.4	E-UTRAN - NR FR1 Handover	197

5.3.4.1	Introduction	197

5.3.4.2	Handover delay	197

5.3.4.3	Interruption time	197

5.3.4A	E-UTRAN - NR FR1 Handover to target cell using CCA	197

5.3.4A.1	Introduction	197

5.3.4A.2	Handover delay	198

5.3.4A.3	Interruption time	198

5.3.5	E-UTRAN - NR FR2 Handover	199

5.3.5.1	Introduction	199

5.3.5.2	Handover delay	199

5.3.5.3	Interruption time	199

5.4	Handover to Non-3GPP RATs	200

5.4.1	E-UTRAN – HRPD Handover	200

5.4.1.1	Introduction	200

5.4.1.1.1	Handover delay	200

5.4.1.1.2	Interruption time	200

5.4.2	E-UTRAN – cdma2000 1X Handover	201

5.4.2.1	Introduction	201

5.4.2.1.1	Handover delay	201

5.4.2.1.2	Interruption time	201

5.5	E-UTRAN Handover for Cat-M1 UEs	201

5.5.1	Introduction	201

5.5.2	Requirements in CEModeA	201

5.5.2.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	201

5.5.2.1.1	Handover delay	202

5.5.2.1.2	Interruption time	202

5.5.2.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	202

5.5.2.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	202

5.5.2.3.1	Void	203

5.5.2.3.2	Void	203

5.5.3	Requirements in CEModeB	203

5.5.3.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	203

5.5.3.1.1	Handover delay	203

5.5.3.1.2	Interruption time	203

5.5.3.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	204

5.5.3.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	204

5.6	Void	204

5.7	E-UTRAN DAPS Handover	204

5.7.1	Introduction	204

5.7.2	Requirements	204

5.7.2.1	E-UTRAN FDD – FDD	204

5.7.2.1.1	DAPS Handover delay	204

5.7.2.1.2	Interruption time	205

5.7.2.2	E-UTRAN FDD – TDD	205

5.7.2.3	E-UTRAN TDD – FDD	205

5.7.2.4	E-UTRAN TDD – TDD	206

6	RRC Connection Mobility Control	206

6.1	RRC Re-establishment	206

6.1.1	Introduction	206

6.1.2	Requirements	206

6.1.2.1	UE Re-establishment delay requirement	206

6.2	Random Access	207

6.2.1	Introduction	207

6.2.2	Requirements	207

6.2.2.1	Contention based random access	207

6.2.2.1.1	Correct behaviour when receiving Random Access Response reception	207

6.2.2.1.2	Correct behaviour when not receiving Random Access Response reception	207

6.2.2.1.3	Correct behaviour when receiving a NACK on msg3	207

6.2.2.1.4	Void	207

6.2.2.1.5	Correct behaviour when receiving a message over Temporary C-RNTI	207

6.2.2.1.6	Correct behaviour when contention Resolution timer expires	207

6.2.2.2	Non-Contention based random access	208

6.2.2.2.1	Correct behaviour when receiving Random Access Response	208

6.2.2.2.2	Correct behaviour when not receiving Random Access Response	208

6.2.3	Requirements for Cat-M1 UEs	208

6.3	RRC Connection Release with Redirection	208

6.3.1	Introduction	208

6.3.2	Requirements	208

6.3.2.1	RRC connection release with redirection to UTRAN FDD	208

6.3.2.2	RRC connection release with redirection to GERAN	209

6.3.2.3	RRC connection release with redirection to UTRAN TDD	209

6.3.2.4	RRC connection release with redirection to NR	210

6.3.2.5	RRC connection release with redirection to NR carrier subject to CCA	210

6.4	CSG Proximity Indication for E-UTRAN and UTRAN	211

6.4.1	Introduction	211

6.4.2	Requirements	212

6.5	RRC Re-establishment for NB-IoT UEs	212

6.5.1	Introduction	212

6.5.2	Requirements	212

6.5.2.1	UE Re-establishment delay requirement in normal coverage	212

6.5.2.2	UE Re-establishment delay requirement in enhanced coverage	213

6.6	Random Access for UE category NB1	213

6.6.1	Introduction	213

6.6.2	Requirements	213

6.6.2.1	Correct behaviour when receiving Random Access Response reception	213

6.6.2.2	Correct behaviour when not receiving Random Access Response reception	214

6.6.2.3	Correct behaviour when receiving a NACK on msg3	214

6.6.2.4	Correct behaviour when receiving a message over Temporary C-RNTI	214

6.6.2.5	Correct behaviour when contention Resolution timer expires	214

6.6.2.6	MSG3-based channel quality report for UE Category NB1	214

6.6.3	Requirements for NPRACH configuration	215

6.7	RRC Re-establishment for Cat-M1 UEs	215

6.7.1	Introduction	215

6.7.2	Requirements	215

6.7.2.1	UE Re-establishment delay requirement for CEModeA	215

6.7.2.2	UE Re-establishment delay requirement for CEModeB	216

6.8	RRC Connection Release with Redirection for Cat-M1 UEs	216

6.8.1	Introduction	216

6.8.2	Requirements	216

6.8.2.1	RRC connection release with redirection to E-UTRAN with CE Mode A	216

6.9	RRC Connection Redirection to Non-anchor Carrier in NB-IoT	217

6.9.1	Introduction	217

6.9.2	Requirements	217

7	Timing and signalling characteristics	218

7.1	UE transmit timing	218

7.1.1	Introduction	218

7.1.2	Requirements	218

7.2	UE timer accuracy	220

7.2.1	Introduction	220

7.2.2	Requirements	220

7.3	Timing Advance	220

7.3.1	Introduction	220

7.3.2	Requirements	220

7.3.2.1	Timing Advance adjustment delay	220

7.3.2.2	Timing Advance adjustment accuracy	221

7.4	Cell phase synchronization accuracy (TDD)	221

7.4.1	Definition	221

7.4.2	Minimum requirements	221

7.5	Synchronization Requirements for E-UTRAN to 1xRTT and HRPD Handovers	221

7.5.1	Introduction	221

7.5.2	eNodeB Synchronization Requirements	222

7.5.2.1	Synchronized E-UTRAN	222

7.5.2.2	Non-Synchronized E-UTRAN	222

7.6	Radio Link Monitoring	222

7.6.1	Introduction	222

7.6.2	Requirements	224

7.6.2.1	Minimum requirement when no DRX is used	224

7.6.2.2	Minimum requirement when DRX is used	224

7.6.2.3	Minimum requirement at transitions	225

7.6.2.4	Minimum requirement during SI Acquisition with autonomous gaps	226

7.6.2.5	Minimum requirement under IDC Interference	226

7.7	SCell Activation and Deactivation Delay for E-UTRA Carrier Aggregation	226

7.7.1	Introduction	226

7.7.2	SCell Activation Delay Requirement for Deactivated SCell	226

7.7.3	SCell Deactivation Delay Requirement for Activated SCell	228

7.7.4	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	229

7.7.5	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	230

7.7.6	SCell Activation Delay Requirement for Deactivated PUCCH SCell	231

7.7.7	SCell Activation Delay Requirement for Deactivated PUCCH SCell with Multiple SCells	232

7.7.8	SCell Deactivation Delay Requirement for Activated PUCCH SCell	232

7.7.9	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple SCells	232

7.7.10	SCell Activation Delay Requirement for Deactivated SCell under Frame Structure 3	233

7.7.11	SCell Deactivation Delay Requirement for Activated SCell under Frame Structure 3	234

7.7.12	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells under Frame Structure 3	235

7.7.13	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells under Frame Structure 3	236

7.7.14	SCell Activation Delay Requirement for Dormant SCell	236

7.7.15	SCell Hibernation Delay Requirement for Activated SCell	237

7.7.16	SCell Hibernation Delay Requirement for Deactivated SCell	238

7.7.17	SCell Deactivation Delay Requirement for Dormant SCell	240

7.7.18	Direct SCell Activation and Hibernation Delay Requirement	240

7.7.19	Direct SCell Activation and Hibernation Delay Requirement at RRC Reconfiguration during Handover	242

7.8	Interruptions with Carrier Aggregation	244

7.8.1	Introduction	244

7.8.2	Requirements	244

7.8.2.1	Interruptions at SCell addition/release for intra-band CA	244

7.8.2.2	Interruptions at SCell addition/release for inter-band CA	244

7.8.2.3	Interruptions at SCell activation/deactivation for intra-band CA	244

7.8.2.4	Interruptions at SCell activation/deactivation for inter-band CA	244

7.8.2.5	Interruptions during measurements on SCC for intra-band CA	245

7.8.2.6	Interruptions during measurements on SCC for inter-band CA	245

7.8.2.7	Interruptions at SCell addition/release with multiple downlink SCells	245

7.8.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	245

7.8.2.9	Interruptions during measurements on SCC with multiple downlink SCells	246

7.8.2.10	Interruptions at overlapping addition/release/activation/deactivation of SCells	247

7.8.2.11	Interruptions during RSSI measurements on one SCC under Frame Structure 3	247

7.8.2.12	Interruptions during RSSI measurements on multiple SCCs under Frame Structure 3	247

7.8.2.13	Interruptions at SRS carrier based switching	248

7.8.2.14	Interruptions at SCell activation and deactivation of dormant SCell for intra-band CA	248

7.8.2.15	Interruptions at SCell activation and deactivation of dormant SCell for inter-band CA	249

7.8.2.16	Interruptions at SCell activation and deactivation of multiple dormant SCells	249

7.8.2.17	Interruptions during CQI measurement on dormant SCell	249

7.8.2.18	Interruptions during RRM measurement on dormant SCell for intra-band CA	249

7.8.2.19	Interruptions during RRM measurement on dormant SCell for inter-band CA	250

7.8.2.20	Interruptions at SCell hibernation	250

7.8.2.21	Interruptions at direct SCell activation and hibernation	250

7.9	Maximum Transmission Timing Difference in Carrier Aggregation	251

7.9.1	Introduction	251

7.9.2	Minimum Requirements for Interband Carrier Aggregation	251

7.9.3	Minimum Requirements for Intraband non-contiguous Carrier Aggregation	251

7.9.4	Minimum Requirements for Inter-Band Carrier Aggregation under Frame Structure 3	252

7.10	Interruptions with RSTD Measurements with Carrier Aggregation	252

7.10.1	Introduction	252

7.10.2	Requirements	252

7.10.2.1	Interruptions during RSTD measurements on SCC for intra-band CA with one downlink SCell	252

7.10.2.2	Interruptions during RSTD measurements on SCC for inter-band CA with one downlink SCell	252

7.10.2.3	Interruptions during RSTD measurements on SCC with multiple downlink SCells	253

7.10.2.4	Interruptions at overlapping RSTD and inter-frequency measurements	253

7.11	Radio Link Monitoring for UE Category 0	253

7.11.1	Introduction	253

7.11.2	Requirements for FD-FDD and TDD	255

7.11.2.1	Minimum requirement when no DRX is used	255

7.11.2.2	Minimum requirement when DRX is used	255

7.11.2.3	Minimum requirement at transitions	256

7.11.3	Requirements for HD-FDD	256

7.11.3.1	Minimum requirement when no DRX is used	256

7.11.3.2	Minimum requirement when DRX is used	256

7.11.3.3	Minimum requirement at transitions	257

7.12	Interruptions with Dual Connectivity	257

7.12.1	Introduction	257

7.12.2	Requirements	257

7.12.2.1	Interruptions at PSCell addition/release	257

7.12.2.2	Interruptions at transitions between active and non-active during DRX	258

7.12.2.3	Interruptions at transitions from non-DRX to DRX	258

7.12.2.4	Interruptions at SCell addition/release	258

7.12.2.5	Interruptions at SCell activation/deactivation	258

7.12.2.6	Interruptions during measurements on SCC	259

7.12.2.7	Interruptions at SRS carrier based switching	259

7.13	Cell phase synchronization accuracy (Synchronized mode of dual connectivity)	260

7.13.1	Definition	260

7.13.2	Minimum requirements	260

7.14	PSCell Addition and Release Delay for E-UTRA Dual Connectivity	260

7.14.1	Introduction	260

7.14.2	PSCell Addition Delay Requirement	260

7.14.3	PSCell Release Delay Requirement	261

7.15	Maximum Receive Timing Difference in Dual Connectivity	261

7.15.1	Introduction	261

7.15.2	Minimum Requirements for Inter-band Dual Connectivity	261

7.16	Proximity-based Services	262

7.16.1	Introduction	262

7.16.2	Requirements	262

7.16.2.1	ProSe UE transmission timing	262

7.16.2.1.1	Serving cell or PCell as timing reference	262

7.16.2.1.2	SCell or non-serving cell as timing reference	262

7.16.3	Interruptions with ProSe	262

7.16.3.1	Interruptions at ProSe Direct Discovery configuration	263

7.16.3.2	Interruptions at ProSe Direct Communication configuration	263

7.16.3.3	Interruptions during ProSe Direct Discovery	263

7.16.3.4	Interruptions during ProSe Direct Discovery with discovery gaps	263

7.16.3.5	Interruptions during ProSe Direct Communication	264

7.16.4	Cell reselection for ProSe Direct Discovery on non-serving frequency	264

7.16.4.1	Measurement and evaluation of selected cell	264

7.16.4.2	Measurement of intra-frequency E-UTRAN cells	264

7.16.5	Selection / Reselection of ProSe relay UE	265

7.16.6	ProSe operation under deactivated SCell	265

7.17	Maximum Transmission Timing Difference in Dual Connectivity	266

7.17.1	Introduction	266

7.17.2	Minimum Requirements for maximum transmission timing difference Inter-band Dual Connectivity	266

7.18.1	Introduction	266

7.18.2	SCell Activation Delay Requirement for Deactivated SCell	266

7.18.3	SCell Deactivation Delay Requirement for Activated SCell	266

7.19	Radio Link Monitoring for UE Category M1	266

7.19.1	Introduction	266

7.19.2	Requirements for FD-FDD and TDD CE mode A	267

7.19.2.1	Minimum requirement when no DRX is used	269

7.19.2.2	Minimum requirement when DRX is used	270

7.19.2.3	Minimum requirement at transitions	271

7.19.3	Requirements for HD-FDD with CE mode A	271

7.19.3.1	Minimum requirement when no DRX is used	271

7.19.3.2	Minimum requirement when DRX is used	271

7.19.3.3	Minimum requirement at transitions	273

7.19.4	Requirements for FD-FDD and TDD with CE mode B	273

7.19.4.1	Minimum requirement when no DRX is used	275

7.19.4.2	Minimum requirement when DRX is used	276

7.19.4.3	Minimum requirement at transitions	277

7.19.5	Requirements for HD-FDD with CE mode B	277

7.19.5.1	Minimum requirement when no DRX is used	277

7.19.5.2	Minimum requirement when DRX is used	277

7.19.5.3	Minimum requirement at transitions	279

7.20	UE transmit timing for NB-IoT	279

7.20.1	Introduction	279

7.20.2	Requirements	279

7.21	 UE timer accuracy for NB-IoT	280

7.21.1	Introduction	280

7.21.2	Requirements	280

7.22	Timing Advance for NB-IoT	280

7.22.1	Introduction	280

7.22.2	Requirements	280

7.22.2.1	Timing Advance adjustment delay	280

7.22.2.2	Timing Advance adjustment accuracy	280

7.23	Radio Link Monitoring for Category NB1 UE	280

7.23.1	Introduction	280

7.23.2	Requirements for Category NB1 UE	280

7.23.2.1	Minimum requirement when no DRX is used	281

7.23.2.2	Minimum requirement when DRX is used	281

7.23.2.3	Minimum requirement at transitions	282

7.24	UE transmit timing for Category M1	282

7.24.1	Introduction	282

7.24.2	Requirements	282

7.25	Cell phase synchronization accuracy for MBMS services (FDD)	283

7.25.1	Definition	283

7.25.2	Minimum requirements	283

7.26	UE transmit timing for Category M2	284

7.26.1	Introduction	284

7.26.2	Requirements	284

7.27	UE timer accuracy for category M1	284

7.27.1	Introduction	284

7.27.2	Requirements	284

7.28	Timing Advance for Category M1	285

7.28.1	Introduction	285

7.28.2	Requirements	285

7.29	Interruptions requirements with FeMBMS	285

7.29.1	Introduction	285

7.29.2	Requirements	285

7.30	Numerology switching delay requirements with FeMBMS	285

7.30.1	Introduction	285

7.30.2	Requirements	285

7.31	NR PSCell Addition and Release Delay for E-UTRA - NR Dual Connectivity	285

7.31.1	Introduction	285

7.31.2	NR PSCell Addition Delay Requirement	286

7.31.3	NR PSCell Release Delay Requirement	286

7.31A	Addition and Release Delay of NR PSCell Operating with CCA for E-UTRA - NR Dual Connectivity	287

7.31A.1	Introduction	287

7.31A.2	NR PSCell Addition Delay Requirement	287

7.31A.3	NR PSCell Release Delay Requirement	288

7.32	Interruptions with EN-DC	288

7.32.1	Introduction	288

7.32.2	Requirements	289

7.32.2.1	Interruptions at PSCell addition/release	289

7.32.2.2	Interruptions at transitions between active and non-active during DRX	289

7.32.2.3	Interruptions at transitions from non-DRX to DRX	289

7.32.2.4	Interruptions at SCell addition/release	289

7.32.2.5	Interruptions at SCell activation/deactivation	289

7.32.2.6	Interruptions during measurements on SCC	290

7.32.2.6.1	Interruptions during measurements on deactivated NR SCC	290

7.32.2.6.2	Interruptions during measurements on deactivated E-UTRA SCC	290

7.32.2.6.3	Interruptions during CQI measurements on dormant E-UTRA SCell	290

7.32.2.6.4	Interruptions during RRM measurements on dormant E-UTRA SCC	290

7.32.2.7	Interruptions at active BWP switching	290

7.32.2.8	Interruptions at SCell activation and deactivation of dormant SCell	291

7.32.2.9	Interruptions at SCell activation and deactivation of multiple dormant SCell	291

7.32.2.10	Interruptions at SCell hibernation	291

7.32.2.11	Interruptions at direct SCell activation and hibernation	292

7.32.2.12	DL Interruptions at UE switching between two uplink carriers	292

7.32.2.13	Interruptions at NR SRS carrier based switching	292

7.32.2.14	Interruptions at NR SCell dormancy	293

7.32.2.14.1	Interruptions due to NR SCell dormancy switch	293

7.32.2.14.2	Interruptions due to CSI and RRM measurements during SCell dormancy	293

7.32.2.15	Interruption during NR measurement with autonomous gaps	293

7.32.2.16	Interruptions at SRS carrier based switching	294

7.33	Maximum Transmit/Receive Timing Difference in Carrier Aggregation for sTTI and 1ms-TTI with 3 subframe HARQ processing	294

7.33.1	Introduction	294

7.33.2	Requirements	294

7.34	Void	295

7.35	Interruptions with SFTD measurements	295

7.35.1	Introduction	295

7.35.2	Requirements	295

7.36	Interruptions with NE-DC	295

7.36.1	Introduction	295

7.36.2	Requirements	296

7.36.2.1	Interruptions at transitions between active and non-active during DRX	296

7.36.2.2	Interruptions at transitions from non-DRX to DRX	296

7.36.2.3	Interruptions at SCell addition/release	296

7.36.2.4	Interruptions at SCell activation/deactivation	296

7.36.2.5	Interruptions during measurements on SCC	297

7.36.2.5.1	Interruptions during measurements on deactivated NR SCC	297

7.36.2.5.2	Interruptions during measurements on deactivated E-UTRA SCC	297

7.36.2.5.3	Interruptions during CQI measurements on dormant E-UTRA SCell	297

7.36.2.5.4	Interruptions during RRM measurements on dormant E-UTRA SCC	297

7.36.2.6	Interruptions at active BWP switching	297

7.36.2.7	Interruptions at SCell activation and deactivation of dormant SCell	298

7.36.2.8	Interruptions at SCell activation and deactivation of multiple dormant SCell	298

7.36.2.9	Interruptions at SCell hibernation	298

7.36.2.10	Interruptions at direct SCell activation and hibernation	298

7.36.2.11	Interruptions at NR SRS carrier based switching	299

7.36.2.12	Interruptions at NR SCell dormancy	299

7.36.2.12.1	Interruptions due to NR SCell dormancy switch	299

7.36.2.12.2	Interruptions due to CSI and RRM measurements during SCell dormancy	300

7.36.2.13	Interruption during E-UTRA measurement with autonomous gaps	300

7.36.2.14	Interruption during NR measurement with autonomous gaps	300

7.36.2.15	Interruptions at SRS carrier based switching	301

7.37	Interruptions during NR measurement with autonomous gaps	301

7.37.1	Introduction	301

7.37.2	Requirements	301

8	UE Measurements Procedures in RRC_CONNECTED State	302

8.1	General Measurement Requirements	302

8.1.1	Introduction	302

8.1.2	Requirements	302

8.1.2.1	UE measurement capability	302

8.1.2.1.1	Monitoring of multiple layers using gaps	309

8.1.2.1.1a	Monitoring of multiple layers using gaps (Increased UE carrier monitoring)	310

8.1.2.1.1b	Monitoring of multiple layers using gaps (EN-DC)	311

8.1.2.1.1c	Monitoring of multiple layers using gaps (NE-DC)	313

8.1.2.1.2	Network controlled small gap	314

8.1.2.2	E-UTRAN intra frequency measurements	315

8.1.2.2.1	E-UTRAN FDD intra frequency measurements	315

8.1.2.2.2	E-UTRAN TDD intra frequency measurements	320

8.1.2.2.3	E-UTRAN FDD intra frequency measurements with autonomous gaps	324

8.1.2.2.4	E-UTRAN TDD intra frequency measurements with autonomous gaps	325

8.1.2.2.5	E-UTRAN FDD intra-frequency measurements on carrier with FeMBMS/Unicast mixed cells	326

8.1.2.3	E-UTRAN inter frequency measurements	327

8.1.2.3.1	E-UTRAN FDD – FDD inter frequency measurements	327

8.1.2.3.2	E-UTRAN TDD – TDD inter frequency measurements	333

8.1.2.3.3	E-UTRAN TDD – FDD inter frequency measurements	339

8.1.2.3.4	E-UTRAN FDD – TDD inter frequency measurements	339

8.1.2.3.5	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	340

8.1.2.3.6	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	341

8.1.2.3.7	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	342

8.1.2.3.8	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	343

8.1.2.3.9	E-UTRAN FDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	344

8.1.2.3.10	E-UTRAN TDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	351

8.1.2.4	Inter RAT measurements	351

8.1.2.4.1	E-UTRAN FDD – UTRAN FDD measurements	351

8.1.2.4.2	E-UTRAN TDD – UTRAN FDD measurements	356

8.1.2.4.3	E-UTRAN TDD – UTRAN TDD measurements	356

8.1.2.4.4	E-UTRAN FDD – UTRAN TDD measurements	360

8.1.2.4.5	E-UTRAN FDD – GSM measurements	360

8.1.2.4.6	E-UTRAN TDD – GSM measurements	365

8.1.2.4.7	E-UTRAN FDD – UTRAN FDD measurements for SON	365

8.1.2.4.8	E-UTRAN TDD – UTRAN FDD measurements for SON	367

8.1.2.4.9	E-UTRAN FDD – cdma2000 1xRTT measurements	367

8.1.2.4.9.1A	E-UTRAN FDD – cdma2000 1xRTT measurements when no DRX is used	367

8.1.2.4.10	E-UTRAN TDD – cdma2000 1xRTT measurements	368

8.1.2.4.11	E-UTRAN FDD – HRPD measurements	368

8.1.2.4.12	E-UTRAN TDD – HRPD measurements	368

8.1.2.4.13	E-UTRAN TDD – UTRAN TDD measurements for SON	368

8.1.2.4.14	E-UTRAN FDD – UTRAN TDD measurements for SON	370

8.1.2.4.15	E-UTRAN FDD – cdma2000 1xRTT measurements for SON ANR	370

8.1.2.4.16	E-UTRAN TDD – cdma2000 1xRTT measurements for SON ANR	370

8.1.2.4.17	E-UTRAN FDD-UTRAN FDD measurements with autonomous gaps	370

8.1.2.4.18	E-UTRAN TDD-UTRAN FDD measurements with autonomous gaps	371

8.1.2.4.19	E-UTRAN FDD – WLAN measurements	371

8.1.2.4.20	E-UTRAN TDD – WLAN measurements	373

8.1.2.4.21	E-UTRAN FDD – NR measurements	373

8.1.2.4.21A	E-UTRAN FDD – NR measurements when CCA is used	377

8.1.2.4.22	E-UTRAN TDD – NR measurements	381

8.1.2.4.22A	E-UTRAN TDD – NR measurements when CCA is used	381

8.1.2.4.23	Void	381

8.1.2.4.24	Void	381

8.1.2.4.25	E-UTRAN FDD – NR SFTD Measurements	381

8.1.2.4.26	E-UTRAN TDD – NR SFTD Measurements	383

8.1.2.4.27 	E-UTRA FDD - NR measurements with autonomous gaps	383

8.1.2.4.28 	E-UTRA TDD - NR measurements with autonomous gaps	384

8.1.2.5	E-UTRAN OTDOA Intra-Frequency RSTD Measurements	384

8.1.2.5.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	385

8.1.2.5.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	386

8.1.2.5.3	E-UTRAN FDD Intra-Frequency OTDOA Measurements for UE Category 1bis	388

8.1.2.5.4	E-UTRAN TDD Intra-Frequency OTDOA Measurements for UE Category 1bis	390

8.1.2.6.5	Void	391

8.1.2.6.6	Void	391

8.1.2.6.7	Void	391

8.1.2.6.8	Void	391

8.1.2.6	E-UTRAN Inter-Frequency OTDOA Measurements	391

8.1.2.6.1	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements	392

8.1.2.6.2	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements	393

8.1.2.6.3	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements	395

8.1.2.6.4	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements	397

8.1.2.6.5	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	399

8.1.2.6.6	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	400

8.1.2.6.7	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	402

8.1.2.6.8	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	404

8.1.2.7	E-UTRAN E-CID Measurements	406

8.1.2.7.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	406

8.1.2.7.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	407

8.1.2.7.3	E-UTRAN FDD Intra-frequency E-CID RSRP and RSRQ Measurements	408

8.1.2.7.4	E-UTRAN TDD Intra-frequency E-CID RSRP and RSRQ Measurements	409

8.1.2.8	E-UTRAN intra-frequency measurements under time domain measurement resource restriction	409

8.1.2.8.1	E-UTRAN FDD intra-frequency measurements	410

8.1.2.8.2	E-UTRAN TDD intra-frequency measurements	412

8.1.2.8.3	E-UTRAN FDD intra-frequency measurements with CRS assistance information	415

8.1.2.8.4	E-UTRAN TDD intra-frequency measurements with CRS assistance infromation	419

8.1.2.9	E-UTRAN E-CID Measurements when Time Domain Measurement Resource Restriction Pattern is Configured	422

8.1.2.9.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	422

8.1.2.9.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	423

8.1.2.9.3	E-UTRAN FDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	423

8.1.2.9.4	E-UTRAN TDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	424

8.1.2.10	Void	424

8.2	Capabilities for Support of Event Triggering and Reporting Criteria	424

8.2.1	Introduction	424

8.2.2	Requirements	424

8.3	Measurements for E-UTRA carrier aggregation	429

8.3.1	Introduction	429

8.3.2	Measurements of the primary component carrier	429

8.3.3	Measurements of a secondary component carrier	429

8.3.3.1	Measurements of a secondary component carrier with active SCell	429

8.3.3.2	Measurements of a secondary component carrier with deactivated SCell	430

8.3.3.2.1	E-UTRAN secondary component carrier measurements when no common DRX is used	430

8.3.3.2.2	E-UTRAN secondary component carrier measurements when common DRX is used	431

8.3.3.3	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and activated SCell	433

8.3.3.4	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and deactivated SCell	433

8.4	OTDOA RSTD Measurements for E-UTRAN carrier aggregation	433

8.4.1	Introduction	433

8.4.2	Measurements on the primary component carrier	434

8.4.3	Measurements on a secondary component carrier	435

8.4.4	Measurements on both primary component carrier and a secondary component carrier	435

8.4.5	Measurements on different secondary component carriers	436

8.5	Measurements for UE category 0	437

8.5.1	Introduction	437

8.5.2 	Requirements	438

8.5.2.1 	E-UTRAN intra frequency measurements	438

8.5.2.1.1 	E-UTRAN FDD intra frequency measurements	438

8.5.2.1.2 	E-UTRAN intra frequency measurements for HD-FDD	441

8.5.2.1.3	E-UTRAN TDD intra frequency measurements	443

8.5.2.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category 0	447

8.5.2.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category 0	448

8.5.2.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category 0	448

8.6	Discovery signal measurements	449

8.6.1	Introduction	449

8.6.2	Requirements for CRS based discovery signal measurements	449

8.6.2.1	E-UTRAN intra frequency measurements	449

8.6.2.1.1	E-UTRAN FDD intra frequency measurements	449

8.6.2.1.2	E-UTRAN TDD intra frequency measurements	452

8.6.2.2	E-UTRAN inter frequency measurements	454

8.6.2.2.1	E-UTRAN FDD – FDD inter-frequency measurements	455

8.6.2.2.2	E-UTRAN TDD – TDD inter frequency measurements	457

8.6.2.2.3	E-UTRAN TDD – FDD inter frequency measurements	460

8.6.2.2.4	E-UTRAN FDD – TDD inter frequency measurements	460

8.6.3	Requirements for CSI-RS based discovery signal measurements	460

8.6.3.1	E-UTRAN intra frequency measurements	460

8.6.3.1.1	E-UTRAN FDD intra frequency measurements	461

8.6.3.1.2	E-UTRAN TDD intra frequency measurements	463

8.6.3.2	E-UTRAN inter frequency measurements	465

8.6.3.2.1	E-UTRAN FDD – FDD inter frequency measurements	466

8.6.3.2.2	E-UTRAN TDD – TDD inter frequency measurements	468

8.6.3.2.3	E-UTRAN TDD – FDD inter frequency measurements	471

8.6.3.2.4	E-UTRAN FDD – TDD inter frequency measurements	471

8.7	Discovery signal measurements for E-UTRA carrier aggregation	471

8.7.1	Introduction	471

8.7.2	Requirements for CRS based discovery signal measurements for E-UTRA carrier aggregation	472

8.7.2.1	Measurements of the primary component carrier	472

8.7.2.2	Measurements of a secondary component carrier	472

8.7.2.3	Measurements of a secondary component carrier with active SCell	472

8.7.2.4	Measurements of a secondary component carrier with deactivated SCell	472

8.7.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	472

8.7.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	473

8.7.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	475

8.7.3.1	Measurements of the primary component carrier	475

8.7.3.2	Measurements of a secondary component carrier	475

8.7.3.3	Measurements of a secondary component carrier with active SCell	475

8.7.3.4	Measurements of a secondary component carrier with deactivated SCell	475

8.7.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	475

8.7.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	477

8.8	Measurements for E-UTRA dual connectivity	478

8.8.1	Introduction	478

8.8.2	Intra-frequency measurements requirements on PCell	478

8.8.3	Intra-frequency measurements requirements on PSCell	479

8.8.4	Inter-frequency and inter-RAT measurement requirements	479

8.8.5	Intra-frequency measurements with autonomous gaps	479

8.8.5.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	479

8.8.5.2	ECGI reporting delay	480

8.8.6	Inter-frequency measurements with autonomous gaps	480

8.8.6.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	480

8.8.6.2	ECGI reporting delay	481

8.8.7	SSTD Measurements	481

8.8.7.1	Introduction	481

8.8.7.2	SSTD Measurement requirements	481

8.8.7.3	SSTD Measurement Reporting Delay	482

8.8.8	Intra-frequency measurements requirements on SCell	482

8.9	MBSFN Measurements	482

8.9.1	Introduction	482

8.9.2	MBSFN RSRP Measurements	482

8.9.3	MBSFN RSRQ Measurements	483

8.9.4	MCH BLER Measurements	483

8.10	Proximity-based Services	483

8.10.1	Introduction	483

8.10.2	Requirements	483

8.10.2.1	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	483

8.10.2.2	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	484

8.11	Discovery Signal Measurements under Operation with Frame Structure 3	484

8.11.1	Introduction	484

8.11.2	CRS based discovery signal measurements	485

8.11.2.1	E-UTRAN intra-frequency measurements	485

8.11.2.1.1	Requirements	485

8.11.2.1.1.1	Requirements when no DRX is used	485

8.11.2.1.1.1.1	Measurement Reporting Requirements	487

8.11.2.1.1.2	Requirements when DRX is used	488

8.11.2.1.1.2.1	Measurement Reporting Requirements	490

8.11.2.2	E-UTRAN inter-frequency measurements	491

8.11.2.2.1	E-UTRAN FDD-FS3 inter-frequency measurements	491

8.11.2.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	495

8.11.3	CSI-RS based discovery signal measurements	495

8.11.3.1	E-UTRAN intra-frequency measurements	495

8.11.3.1.1	Requirements	495

8.11.3.1.1.1	Requirements when no DRX is used	495

8.11.3.1.1.1.1	Measurement Reporting Requirements	497

8.11.3.1.1.2	Requirements when DRX is used	497

8.11.3.1.1.2.1	Measurement Reporting Requirements	498

8.11.3.2	E-UTRAN inter-frequency measurements	499

8.11.3.2.1	E-UTRAN FDD – FS3 inter-frequency measurements	499

8.11.3.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	503

8.11.4	RSSI measurements	503

8.11.4.1	E-UTRAN intra-frequency measurements	503

8.11.4.2	E-UTRAN inter-frequency measurements	503

8.11.5	Channel occupancy measurements	504

8.11.5.1	E-UTRAN intra-frequency channel occupancy measurements	504

8.11.5.2	E-UTRAN inter-frequency channel occupancy measurements	504

8.12	Discovery Signal Measurements for E-UTRA Carrier Aggregation under Operation with Frame Structure 3	504

8.12.1	Introduction	504

8.12.2	CRS based discovery signal measurements for E-UTRA carrier aggregation	504

8.12.2.1	Introduction	504

8.12.2.2	Measurements of a secondary component carrier	504

8.12.2.3	Measurements of a secondary component carrier with active SCell	504

8.12.2.4	Measurements of a secondary component carrier with deactivated SCell	505

8.12.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	505

8.12.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	507

8.12.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	510

8.12.3.1	Introduction	510

8.12.3.2	Measurements of a secondary component carrier	510

8.12.3.3	Measurements of a secondary component carrier with active SCell	510

8.12.3.4	Measurements of a secondary component carrier with deactivated SCell	510

8.12.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	510

8.12.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	512

8.13	Measurements for UE Category M1	514

8.13.1	Introduction	514

8.13.2	Requirements for UE category M1 with CE mode A	514

8.13.2.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode A	515

8.13.2.1.1	E-UTRAN FDD intra frequency measurements	515

8.13.2.1.2	E-UTRAN intra frequency measurements for HD-FDD	519

8.13.2.1.3	E-UTRAN TDD intra frequency measurements	522

8.13.2.2	Void	526

8.13.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	526

8.13.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	526

8.13.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	529

8.13.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	531

8.13.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	532

8.13.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	532

8.13.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	534

8.13.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	537

8.13.2.5	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode A	537

8.13.2.5.1	Intra-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	537

8.13.2.5.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	537

8.13.2.5.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	538

8.13.2.5.4	Inter-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	538

8.13.2.5.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	539

8.13.2.5.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	539

8.13.2.5.7	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	539

8.13.2.5.8	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	540

8.13.2.5.9	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	541

8.13.2.6	E-UTRAN inter frequency measurements by UE category M1 with CE mode A	542

8.13.2.6.1	E-UTRAN FDD - FDD inter frequency measurements	542

8.13.2.6.2	E-UTRAN inter-frequency measurements for HD-FDD	546

8.13.2.6.3	E-UTRAN TDD inter frequency measurements	548

8.13.2.7	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode A	552

8.13.2.8	Channel quality report for UE Category M1 in connected mode with CE mode A	552

8.13.3	Requirements for UE category M1 with CE mode B	553

8.13.3.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode B	553

8.13.3.1.1	E-UTRAN FDD intra frequency measurements	554

8.13.3.1.2	E-UTRAN intra frequency measurements for HD-FDD	558

8.13.3.1.3	E-UTRAN TDD intra frequency measurements	561

8.13.3.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	566

8.13.3.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category M1 with CE mode B	567

8.13.3.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	567

8.13.3.2	Void	568

8.13.3.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	568

8.13.3.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	568

8.13.3.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	571

8.13.3.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	573

8.13.3.4	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode B	574

8.13.3.4.1	Intra-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	574

8.13.3.4.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	574

8.13.3.4.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	575

8.13.3.4.4	Inter-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	575

8.13.3.4.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	575

8.13.3.4.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	576

8.13.3.5	E-UTRAN inter frequency measurements by UE category M1 with CE Mode B	576

8.13.3.5.1	E-UTRAN FDD - FDD inter frequency measurements	576

8.13.3.5.2 	E-UTRAN inter-frequency measurements for HD-FDD	581

8.13.3.5.3	E-UTRAN TDD inter frequency measurements	583

8.13.3.6	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode B	587

8.13.3.7	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	588

8.13.3.7.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	588

8.13.3.7.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	590

8.13.3.7.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	592

8.13.3.8	Channel quality report for UE Category M1 in connected mode with CE mode B	593

8.14	Measurements for UE category NB1	593

8.14.1	Introduction	593

8.14.2	NB-IoT intra frequency measurements under normal coverage	594

8.14.2.1	NB-IoT intra frequency measurements when no DRX is used	594

8.14.2.2	NB-IoT intra frequency measurements when DRX is used	594

8.14.3	NB-IoT intra frequency measurements under enhanced coverage	594

8.14.3.1	NB-IoT intra frequency measurements when no DRX is used	594

8.14.3.2	NB-IoT intra frequency measurements when DRX is used	594

8.14.4	Connected mode channel quality report for UE Category NB1	594

8.15	Void	595

8.16	Measurements for UE Category M2	595

8.16.1	Introduction	595

8.16.2	Requirements for UE category M2 with CE mode A	595

8.16.2.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	595

8.16.2.1.1	UE Rx-Tx Measurement Reporting Delay	596

8.16.2.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	596

8.16.2.2.1	UE Rx-Tx Measurement Reporting Delay	597

8.16.2.2a	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	597

8.16.2.2a.1	UE Rx-Tx Measurement Reporting Delay	597

8.16.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	598

8.16.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	598

8.16.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	600

8.16.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	603

8.16.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	603

8.16.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	603

8.16.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	606

8.16.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	608

8.16.3	Requirements for UE category M2 with CE mode B	609

8.16.3.1	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	609

8.16.3.1.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	609

8.16.3.1.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	611

8.16.3.1.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	614

8.16.3.2	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	614

8.16.3.2.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	615

8.16.3.2.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	617

8.16.3.2.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	619

8.17	Measurements for E-UTRA – NR Dual Connectivity	620

8.17.1	Introduction	620

8.17.1.1	Measurement Gap Sharing	620

8.17.1A	Intrafrequency Measurements	620

8.17.2	SFTD Measurements	621

8.17.2.1	Introduction	621

8.17.2.2	SFTD Measurement requirements	621

8.17.2.2.a	SFTD Measurement requirements with CCA on target frequency	621

8.17.2.3	SFTD Measurement Reporting Delay	623

8.17.3	E-UTRA Inter-frequency Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	623

8.17.3.1	Introduction	623

8.17.3.2	E-UTRAN FDD inter frequency measurements	623

8.17.3.2.1	E-UTRAN FDD inter frequency measurements when no DRX is used	623

8.17.3.2.2	E-UTRAN FDD inter frequency measurements when DRX is used	625

8.17.3.3	E-UTRAN TDD inter frequency measurements	627

8.17.3.3.1	E-UTRAN TDD inter frequency measurements when no DRX is used	627

8.17.3.3.2	E-UTRAN TDD inter frequency measurements when DRX is used	628

8.17.4	E-UTRA Inter-RAT NR Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	630

8.17.4.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	630

8.17.4.1.1	NR Inter-RAT cell identification	631

8.17.4.1.2	NR Inter-RAT measurement	632

8.17.4.1.3	NR Inter-RAT measurement reporting	633

8.17.4.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	634

8.17.4A	E-UTRA Inter-RAT NR Measurements when CCA is used when Configured with E-UTRA-NR Dual Connectivity Operation	634

8.17.4A.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	634

8.17.4A.1.1	NR Inter-RAT cell identification	635

8.17.4A.1.2	NR Inter-RAT measurement	636

8.17.4A.1.3	NR Inter-RAT measurement reporting	637

8.17.4A.1.4	NR inter-RAT RSSI measurements	637

8.17.4A.1.5	NR inter-RAT channel occupancy measurements	638

8.17.4A.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	638

8.17.5	E-UTRAN FDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	638

8.17.5.1	Introduction	638

8.17.5.2	E-UTRAN FDD – UTRAN FDD measurements when no DRX is used	638

8.17.5.2.1	Identification of a new UTRA FDD cell	638

8.17.5.2.2	Enhanced UTRA FDD cell identification requirements	639

8.17.5.2.3	UE UTRA FDD CPICH measurement capability	639

8.17.5.2.4	Periodic Reporting	639

8.17.5.2.5	Event Triggered Reporting	639

8.17.5.2.6	Event-triggered Periodic Reporting	640

8.17.5.3	E-UTRAN FDD – UTRAN FDD measurements when DRX is used	640

8.17.5.3.1	Periodic Reporting	641

8.17.5.3.2	Event Triggered Reporting	641

8.17.5.3.3	Event-triggered Periodic Reporting	642

8.17.6	E-UTRAN TDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	642

8.17.7	E-UTRAN FDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	642

8.17.7.1	Introduction	642

8.17.7.2	Identification of a new UTRA FDD cell for SON	642

8.17.7.2.1	Requirements when no DRX is used	642

8.17.7.2.2	Requirements when DRX is used	643

8.17.7.2.3	Reporting Delay	643

8.17.8	E-UTRAN TDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	644

8.17.9	E-UTRAN TDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	644

8.17.9.1	Introduction	644

8.17.9.2	E-UTRAN FDD – UTRAN TDD measurements when no DRX is used	644

8.17.9.2.1	Identification of a new UTRA FDD cell	644

8.17.9.2.2	Enhanced UTRA TDD cell identification requirements	644

8.17.9.2.3	UE UTRA TDD P-CCPCH RSCP measurement capability	645

8.17.9.2.4	Periodic Reporting	645

8.17.9.2.5	Event Triggered Reporting	645

8.17.9.2.6	Event-triggered Periodic Reporting	646

8.17.9.3	E-UTRAN TDD – UTRAN TDD measurements when DRX is used	646

8.17.9.3.1	Periodic Reporting	647

8.17.9.3.2	Event Triggered Reporting	647

8.17.9.3.3	Event-triggered Periodic Reporting	647

8.17.10	E-UTRAN FDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	648

8.17.11	E-UTRAN TDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	648

8.17.11.1	Introduction	648

8.17.11.2	Identification of a new UTRA TDD cell for SON	648

8.17.11.2.1	Requirements when no DRX is used	648

8.17.11.2.2	Requirements when DRX is used	648

8.17.11.2.3	Reporting Delay	649

8.17.12	E-UTRAN FDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	649

8.17.13	E-UTRAN FDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	650

8.17.13.1	Introduction	650

8.17.13.2	E-UTRAN FDD – GSM measurements when no DRX is used	650

8.17.13.2.1	GSM carrier RSSI	650

8.17.13.2.2	BSIC verification	650

8.17.13.2.3	Enhanced BSIC verification	652

8.17.13.2.4	Periodic Reporting	652

8.17.13.2.5	Event Triggered Reporting	652

8.17.13.2.6	Event-triggered Periodic Reporting	653

8.17.13.3	E-UTRAN FDD – GSM measurements when DRX is used	653

8.17.13.3.1	GSM carrier RSSI	653

8.17.13.3.2	BSIC verification	653

8.17.13.3.3	Periodic Reporting	655

8.17.13.3.4	Event Triggered Reporting	655

8.17.13.3.5	Event-triggered Periodic Reporting	655

8.17.14	E-UTRAN TDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	655

8.17.15	E-UTRAN Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	655

8.17.15.1	E-UTRAN FDD-FDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	655

8.17.15.1.1	RSTD Measurement Reporting Delay	656

8.17.15.2	E-UTRAN TDD-FDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	656

8.17.15.2.1	RSTD Measurement Reporting Delay	656

8.17.15.3	E-UTRAN TDD-TDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	656

8.17.15.3.1	RSTD Measurement Reporting Delay	657

8.17.15.4	E-UTRAN FDD-TDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	657

8.17.15.4.1	RSTD Measurement Reporting Delay	657

8.17.16	E-UTRAN intra-frequency measurement with autonomous gaps when configured with E-UTRA-NR Dual Connectivity	658

8.17.16.1	Introduction	658

8.17.16.2	E-UTRAN FDD intra frequency measurements with autonomous gaps	658

8.17.16.3	E-UTRAN TDD intra frequency measurements with autonomous gaps	658

8.17.17	E-UTRAN inter-frequency measurement with autonomous gaps when configured with E-UTRA-NR Dual Connectivity	658

8.17.17.1	Introduction	658

8.17.17.2	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	658

8.17.17.3	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	658

8.17.17.4	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	658

8.17.17.5	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	658

8.17.18	E-UTRA FDD - NR CGI measurements with autonomous gaps	658

8.17.19	E-UTRA TDD - NR CGI measurements with autonomous gaps	658

8.18	Measurements for non-BL/CE UE	658

8.18.1	Introduction	658

8.18.2	Requirements for non-BL/CE UE with CE Mode B	659

8.18.2.1	E-UTRAN intra frequency measurements	659

8.18.2.1.1	E-UTRAN FDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	659

8.18.2.1.2	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD non-BL/CE with CE Mode B	659

8.18.2.1.3	E-UTRAN TDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	660

8.19	Measurements for NR – E-UTRA Dual Connectivity	661

8.19.1	Introduction	661

8.19.2	Intra-frequency Measurements	661

8.19.3	Inter-frequency Measurements	661

8.19.4	Void	661

8.19.5	Intra-frequency E-CID Measurements	661

8.19.6	Intra-frequency measurements with autonomous gaps	662

8.19.6.1	Introduction	662

8.19.6.2	E-UTRAN FDD intra frequency measurements with autonomous gaps	662

8.19.6.3	E-UTRAN TDD intra frequency measurements with autonomous gaps	662

8.19.7	Inter-frequency measurements with autonomous gaps	662

8.19.7.1	Introduction	662

8.19.7.2	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	662

8.19.7.3	E-UTRAN TDD-FDD inter frequency measurements with autonomous gaps	662

8.19.7.4	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	662

8.19.7.5	E-UTRAN FDD-TDD inter frequency measurements with autonomous gaps	662

9	Measurements performance requirements for UE	662

9.1	E-UTRAN measurements	662

9.1.1	Introduction	662

9.1.2	Intra-frequency RSRP Accuracy Requirements	663

9.1.2.1	Absolute RSRP Accuracy	663

9.1.2.2	Relative Accuracy of RSRP	663

9.1.2.3	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction	664

9.1.2.4	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction	665

9.1.2.5	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	666

9.1.2.6	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction with CRS assistance information	667

9.1.2.7	Absolute RSRP Accuracy for UE Category 1bis	668

9.1.2.8	Relative Accuracy of RSRP for UE Category 1bis	669

9.1.2A	Intra-frequency RSRP Accuracy Requirements in High Doppler Conditions	670

9.1.2A.1	Absolute RSRP Accuracy in high Doppler conditions	670

9.1.2A.2	Relative Accuracy of RSRP in high Doppler conditions	670

9.1.2B	Intra-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	671

9.1.2B.1	Introduction	671

9.1.2B.2	Intra-frequency Absolute RSRP Accuracy for CA Idle Mode Measurements	671

9.1.3	Inter-frequency RSRP Accuracy Requirements	672

9.1.3.1	Absolute RSRP Accuracy	672

9.1.3.2	Relative Accuracy of RSRP	673

9.1.3.3	Absolute RSRP Accuracy for UE Category 1bis	674

9.1.3.4	Relative Accuracy of RSRP for UE Category 1bis	674

9.1.3A	Inter-frequency RSRP Accuracy Requirements in High Doppler Conditions	675

9.1.3A.1	Absolute RSRP Accuracy in high Doppler conditions	675

9.1.3A.2	Relative Accuracy of RSRP in high Doppler conditions	676

9.1.3B	Inter-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	677

9.1.3B.1	Introduction	677

9.1.3B.2	Inter-frequency Absolute RSRP Accuracy for Overlapping Carrier	677

9.1.3B.3	Inter-frequency Absolute RSRP Accuracy for Overlapping and Non-overlapping Carrier	677

9.1.4	RSRP Measurement Report Mapping	678

9.1.5	Intra-frequency RSRQ Accuracy Requirements	678

9.1.5.1	Absolute RSRQ Accuracy	678

9.1.5.2	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction	679

9.1.5.3	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	680

9.1.5.4	Absolute WB-RSRQ Accuracy	681

9.1.5.5	Absolute RSRQ Accuracy for UE Category 1bis	682

9.1.5A	Intra-frequency RSRQ Accuracy Requirements in High Doppler Conditions	683

9.1.5A.1	Absolute RSRQ Accuracy in high Doppler conditions	683

9.1.5B	Intra-frequency RSRQ Accuracy requirements for CA Idle Mode Measurements	683

9.1.5B.1	Introduction	683

9.1.5B.2	Intra-frequency Absolute RSRQ Accuracy for CA Idle Mode Measurements	683

9.1.6	Inter-frequency RSRQ Accuracy Requirements	684

9.1.6.1	Absolute RSRQ Accuracy	684

9.1.6.2	Relative Accuracy of RSRQ	685

9.1.6.3	Absolute WB-RSRQ Accuracy	686

9.1.6.4	Relative WB-RSRQ Accuracy	686

9.1.6.5	Absolute RSRQ Accuracy for UE Category 1bis	687

9.1.6.6	Relative Accuracy of RSRQ for UE Category 1bis	688

9.1.6A	Inter-frequency RSRQ Accuracy Requirements in High Doppler Conditions	688

9.1.6A.1	Absolute RSRQ Accuracy in high Doppler conditions	688

9.1.6A.2	Relative Accuracy of RSRQ in high Doppler conditions	689

9.1.6B	Inter-frequency absolute RSRQ Accuracy requirements for CA Idle Mode Measurements	690

9.1.6B.1	Introduction	690

9.1.6B.2	Inter-frequency Absolute RSRQ Accuracy for Overlapping Carrier	690

9.1.6B.3	Inter-frequency absolute RSRQ Accuracy for Overlapping and Non-overlapping Carrier	690

9.1.7	RSRQ Measurement Report Mapping	691

9.1.8	Power Headroom	691

9.1.8.1	Period	692

9.1.8.2	Reporting Delay	692

9.1.8.3	Void	692

9.1.8.4	Report Mapping	692

9.1.9	UE Rx – Tx time difference	692

9.1.9.1	Measurement Requirement	692

9.1.9.2	Measurement Report mapping	693

9.1.9.3	Measurement Requirement under Time Domain Measurement Resource Restriction	694

9.1.9.4	Measurement Requirement when Time Domain Measurement Resource Restriction Pattern is Configured with CRS Assistance Information	695

9.1.10	Reference Signal Time Difference (RSTD)	696

9.1.10.1	Intra-Frequency Accuracy Requirement	696

9.1.10.2	Inter-Frequency Accuracy Requirement	697

9.1.10.3	RSTD Measurement Report Mapping	699

9.1.10.4	Higher-Resolution RSTD Measurement Report Mapping	699

9.1.10.5	Intra-Frequency Accuracy Requirement for UE Category 1bis	700

9.1.10.6	Inter-Frequency Accuracy Requirement for UE Category 1bis	701

9.1.11	Carrier aggregation measurement accuracy	702

9.1.11.1	Primary component carrier accuracy requirement	703

9.1.11.2	Secondary component carrier accuracy requirement	703

9.1.11.3	Primary and secondary component carrier relative accuracy requirement	703

9.1.11.4	Secondary component carrier relative accuracy requirement	703

9.1.12	Reference Signal Time Difference (RSTD) Measurement Accuracy Requirements for Carrier Aggregation	703

9.1.13	Measurement accuracy for UE category 0	704

9.1.13.1	Intra-frequency Absolute RSRP Accuracy for UE category 0	704

9.1.13.2	Intra-frequency Relative Accuracy of RSRP for UE category 0	704

9.1.13.3	Intra-frequency Absolute RSRQ Accuracy for UE category 0	705

9.1.14	Accuracy requirements for Discovery Signal Measurements	706

9.1.14.1	Introduction	706

9.1.14.2	RSRP measurements in discovery signal occasions	706

9.1.14.3	CSI-RSRP measurements in discovery signal occasions	706

9.1.14.3.1	Intra-frequency CSI-RSRP measurements	706

9.1.14.3.1.1	Absolute CSI-RSRP measurement requirements	706

9.1.14.3.1.2	Relative CSI-RSRP measurement requirements	707

9.1.14.3.2	Inter-frequency CSI-RSRP measurements	708

9.1.14.3.2.1	Absolute CSI-RSRP measurement requirements	708

9.1.14.3.2.2	Relative CSI-RSRP measurement requirements	708

9.1.14.3.3	CSI-RSRP measurement report mapping	709

9.1.14.4	RSRQ measurements in discovery signal occasions	709

9.1.15	Discovery signal measurements accuracy for E-UTRAN carrier aggregation	709

9.1.15.1	Requirements for CRS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	710

9.1.15.1.1	Primary component carrier accuracy requirement	710

9.1.15.1.2	Secondary component carrier accuracy requirement	710

9.1.15.1.3	Primary and secondary component carrier relative accuracy requirement	710

9.1.15.1.4	Secondary component carrier relative accuracy requirement	710

9.1.15.2	Requirements for CSI-RS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	710

9.1.15.2.1	Primary component carrier accuracy requirement	710

9.1.15.2.2	Secondary component carrier accuracy requirement	710

9.1.15.2.3	Primary and secondary component carrier relative accuracy requirement	710

9.1.15.2.4	Secondary component carrier relative accuracy requirement	710

9.1.16	Accuracy requirements for RSRQ measurement on all OFDM symbols	711

9.1.17	RS-SINR Measurements	711

9.1.17.1	Measurement Report Mapping	711

9.1.17.2	Intra-frequency RS-SINR Measurement Accuracy Requirements	712

9.1.17.2.1	Absolute RS-SINR Measurement Accuracy Requirements	712

9.1.17.3	Inter-frequency RS-SINR Measurement Accuracy Requirements	712

9.1.17.3.1	Absolute RS-SINR Measurement Accuracy Requirements	712

9.1.17.3.2	Relative RS-SINR Measurement Accuracy Requirements	713

9.1.18	Accuracy Requirements for Measurements under Operation with Frame Structure 3	714

9.1.18.1	Introduction	714

9.1.18.2	RSRP measurements	714

9.1.18.2.1	RSRP measurement report mapping	714

9.1.18.2.2	Inter-frequency absolute RSRP measurement accuracy requirements	714

9.1.18.2.3	Inter-frequency relative RSRP measurement accuracy requirements	715

9.1.18.2.4	Intra-frequency absolute RSRP measurement accuracy requirements	715

9.1.18.2.5	Intra-frequency relative RSRP measurement accuracy requirements	716

9.1.18.3	RSRQ measurements	716

9.1.18.3.1	RSRQ measurement report mapping	716

9.1.18.3.2	Inter-frequency absolute RSRQ measurement accuracy requirements	716

9.1.18.3.3	Inter-frequency relative RSRQ measurement accuracy requirements	717

9.1.18.3.4	Intra-frequency absolute RSRQ measurement accuracy requirements	717

9.1.18.4	CSI-RSRP measurements	718

9.1.18.4.1	CSI-RSRP measurement report mapping	718

9.1.18.4.2	Inter-frequency absolute CSI-RSRP measurement accuracy requirements	718

9.1.18.4.3	Inter-frequency relative CSI-RSRP measurement accuracy requirements	718

9.1.18.4.4	Intra-frequency absolute CSI-RSRP measurement accuracy requirements	719

9.1.18.4.5	Intra-frequency relative CSI-RSRP measurement accuracy requirements	719

9.1.18.5	RSSI measurements	720

9.1.18.5.1	RSSI measurement report mapping	720

9.1.18.5.2	Intra-frequency absolute RSSI measurement accuracy requirements	720

9.1.18.5.3	Inter-frequency absolute RSSI measurement accuracy requirements	721

9.1.18.6	Channel occupancy measurements	721

9.1.18.6.1	Intra-frequency channel occupancy measurement accuracy requirements	721

9.1.18.6.2	Inter-frequency channel occupancy measurement accuracy requirements	721

9.1.19	Accuracy Requirements for Carrier Aggregation for Measurements under Operation with Frame Structure 3	722

9.1.19.1	Introduction	722

9.1.19.2	Accuracy requirements for measurements on SCC	722

9.1.19.3	Relative accuracy requirements for measurements on different SCCs	722

9.1.19.4	Relative accuracy requirements for measurements on SCC and PCC	722

9.1.20	SFN and Subframe Time Difference (SSTD)	723

9.1.20.1	SSTD Accuracy Requirement	723

9.1.20.2	SSTD Measurement Report Mapping	723

9.1.21	Measurement accuracy for UE category M1	724

9.1.21.1	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	724

9.1.21.2	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	726

9.1.21.3	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	727

9.1.21.4	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	729

9.1.21.5	RSRP Measurement Report Mapping	730

9.1.21.6	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode A	730

9.1.21.7	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode B	731

9.1.21.8	RSRQ Measurement Report Mapping	732

9.1.21.9	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	732

9.1.21.10	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	733

9.1.21.11	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	734

9.1.21.12	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	735

9.1.21.13 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode A	736

9.1.21.14	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode A	737

9.1.21.15 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode B	738

9.1.21.16	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode B	739

9.1.21.17	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	740

9.1.21.18	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	741

9.1.21.19	UE RX-TX time difference Accuracy Requirement for Cat-M1	742

9.1.21.20	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	743

9.1.21.21	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	746

9.1.21.22	Downlink Channel Report Mapping for UE Category M1	749

9.1.21.23	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode A	749

9.1.21.24	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode B	752

9.1.22	Measurement accuracy for UE Category NB1	753

9.1.22.1	Intra-frequency Absolute NRSRP Accuracy for UE Category NB1	753

9.1.22.2	Void	754

9.1.22.3	Intra-frequency Absolute NRSRQ Accuracy for UE Category NB1	754

9.1.22.4	Void	755

9.1.22.5	Inter-frequency Absolute NRSRP Accuracy for UE Category NB1	755

9.1.22.6	Void	756

9.1.22.7	Inter-frequency Absolute NRSRQ Accuracy for UE Category NB1	756

9.1.22.8	Void	757

9.1.22.9	NRSRP Measurement Report Mapping	757

9.1.22.10	Intra-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	757

9.1.22.11	Inter-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	758

9.1.22.12	Intra-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	759

9.1.22.13	Inter-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	760

9.1.22.14	NRSRQ Measurement Report Mapping	761

9.1.22.15	MSG3-based Measurement Report Mapping for UE Category NB1	762

9.1.22.16	Downlink Channel Quality Measurement Accuracy for UE Category NB1	762

9.1.23	Power Headroom for UE Category NB1	763

9.1.23.1	Period	763

9.1.23.2	Reporting Delay	763

9.1.23.3	Report Mapping for UE Category NB1	763

9.1.23.3.1	Void	765

9.1.23.3.2	Void	765

9.1.23.4	Report Mapping for UE Category NB1 for UE Power Class 6	765

9.1.24	Void	766

9.1.25	Measurement accuracy for UE category M2	766

9.1.25.1	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	766

9.1.25.2	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	767

9.1.25.3	UE RX-TX time difference Accuracy Requirement for Cat-M2	768

9.1.25.4	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	769

9.1.25.5	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	771

9.1.26	Measurement Accuracy for non-BL CE UE	772

9.1.26.1	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	773

9.1.26.2	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	775

9.1.26.3	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	775

9.1.26.4	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	777

9.1.26.5	RSRP Measurement Report Mapping	778

9.1.26.6	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	778

9.1.26.7	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	778

9.1.26.8	RSRQ Measurement Report Mapping	779

9.1.26.9	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	779

9.1.26.10	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	780

9.1.26.11	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	781

9.1.26.12	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	782

9.1.26.13	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	783

9.1.26.14	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode A	783

9.1.26.15	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	783

9.1.26.16	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode B	784

9.1.27	SFN and frame Timing Difference (SFTD)	785

9.1.27.1	SFTD Accuracy Requirement	785

9.2	UTRAN FDD Measurements	787

9.2.1	UTRAN FDD CPICH RSCP	787

9.2.2	Void	788

9.2.3	UTRAN FDD CPICH Ec/No	788

9.3	UTRAN TDD Measurements	788

9.3.1	UTRAN TDD P-CCPCH RSCP	789

9.3.2	Void	789

9.3.3	Void	789

9.4	GSM Measurements	789

9.4.1	GSM carrier RSSI	789

9.5	CDMA2000 1x RTT Measurements	789

9.5.1	CDMA2000 1x RTT Pilot Strength	789

9.6	PCMAX,c	790

9.6.1	Report Mapping	790

9.6.2 	Estimation Period	790

9.6.3	Reporting Delay	790

9.7	IEEE802.11 Measurements	790

9.7.1	WLAN RSSI	790

9.7.2	WLAN RSSI Measurement Report Mapping	790

9.8	MBSFN Measurements	791

9.8.1	Introduction	791

9.8.2	MBSFN RSRP	791

9.8.2.1	Absolute MBSFN RSRP measurement accuracy requirements	791

9.8.2.2	MBSFN RSRP measurement report mapping	792

9.8.2.3	MBSFN RSRP measurement report mapping for 7.5 kHz subcarrier spacing	792

9.8.2.4	MBSFN RSRP measurement report mapping for 1.25 kHz subcarrier spacing	793

9.8.2.5	MBSFN RSRP measurement report mapping for 2.5 kHz subcarrier spacing	793

9.8.2.6	MBSFN RSRP measurement report mapping for 370.37Hz subcarrier spacing	793

9.8.3	MBSFN RSRQ	794

9.8.3.1	Absolute MBSFN RSRQ measurement accuracy requirements	794

9.8.3.2	MBSFN RSRQ measurement report mapping	794

9.8.3.3	MBSFN RSRQ measurement report mapping for 7.5 kHz subcarrier spacing	795

9.8.3.4	MBSFN RSRQ measurement report mapping for 1.25 kHz subcarrier spacing	795

9.8.3.5	MBSFN RSRQ measurement report mapping for 2.5 kHz subcarrier spacing	795

9.8.3.6	MBSFN RSRQ measurement report mapping for 370.37 kHz subcarrier spacing	796

9.8.4	MCH BLER	796

9.8.4.1	Measurement report mapping for MCH BLER	796

9.8.4.2	Measurement report mapping for MCH Block Number	797

9.9	ProSe Measurements	798

9.9.1	Introduction	798

9.9.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	798

9.9.2.1	Absolute S-RSRP Accuracy	798

9.9.2.2	Relative Accuracy of S-RSRP	799

9.9.3	Intra-Frequency SD-RSRP Measurement Accuracy Requirements	800

9.9.3.1	Absolute SD-RSRP Accuracy	800

9.9.3.2	Relative Accuracy of SD-RSRP	800

9.10	V2X Measurements	801

9.10.1	Introduction	801

9.10.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	801

9.10.2.1	Absolute S-RSRP Accuracy	801

9.10.2.2	Relative Accuracy of S-RSRP	802

9.10.3	PSSCH-RSRP Measurement Accuracy Requirements	802

9.10.3.1	Intra-frequency Absolute PSSCH-RSRP Accuracy	802

9.10.4	S-RSSI Measurement Accuracy Requirements	803

9.10.4.1	Intra-frequency absolute S-RSSI measurement accuracy requirements	803

9.10.4.2 Intra-frequency relative S-RSSI measurement accuracy requirements	803

9.11	NR Measurements	804

9.11.1	NR SS-RSRP Measurements	804

9.11.2	NR SS-RSRQ Measurements	804

9.11.3	NR SS-SINR Measurements	805

10	Measurements Performance Requirements for E-UTRAN	805

10.1	Received Interference Power	805

10.1.1	Absolute accuracy requirement	805

10.1.2	Relative accuracy requirement	806

10.1.3	Received Interference Power measurement report mapping	806

10.2	Angle of Arrival (AOA)	806

10.2.1	Range/mapping	806

10.3	Timing Advance (TADV)	806

10.3.1	Report mapping	806

11	ProSe Requirements in Any Cell Selection state	807

11.1	Introduction	807

11.2	UE Transmit Timing for ProSe in Any Cell Selection State	807

11.2.1	Introduction	807

11.2.2	ProSe UE transmission timing	807

11.3	Initiation/Cease of SLSS Transmissions	808

11.3.1	Introduction	808

11.3.2	Requirements	808

11.4	Measurements for ProSe in Any Cell Selection State	808

11.4.1	Introduction	808

11.4.2	Requirements	808

11.4.2.1	E-UTRA FDD	808

11.4.2.2	E-UTRA TDD	809

11.5	Selection / Reselection of ProSe Synchronization Reference	809

11.5.1	Introduction	809

11.5.2	Selection/Reselection to intra-frequency SyncRef UE	809

11.5.2.1	Introduction	809

11.5.2.2	Requirements	809

11.6	Void	810

11.7	Selection / Reselection of ProSe relay UE	810

11.7.1	Introduction	810

11.7.2	Selection / Reselection of intra-frequency ProSe relay UE	810

12	V2V Sidelink Communication Requirements for V2V Operation on Dedicated V2V Carrier	810

12.1	Introduction	810

12.2	Transmit Timing	811

12.2.1	GNSS as timing reference	811

12.3	Interruption	811

12.4	Reliability of GNSS signal	811

13	V2X Requirements	811

13.1	Introduction	811

13.2	UE Transmit Timing	812

13.2.1	Introduction	812

13.2.2	GNSS as synchronization reference source	812

13.2.3	Serving cell/PCell as synchronization reference source	812

13.2.4	SyncRef UE as synchronization reference source	812

13.3	Initiation/Cease of SLSS Transmissions	812

13.3.1	Introduction	812

13.3.1.1	Initiation/Cease of SLSS transmissions with Serving cell / PCell as synchronization reference source	812

13.3.1.2	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	813

13.3.1.3	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	813

13.4	Selection / Reselection of V2X Synchronization Reference Source	814

13.5	Autonomous Resource Selection/Reselection measurements	815

13.5.1	Introduction	815

13.5.2	PSSCH-RSRP measurements	815

13.5.3	S-RSSI measurements	815

13.6	Congestion Control measurements	815

13.7	Interruption	815

13.7.1	Interruptions to WAN due to V2X Sidelink Communication	815

13.7.2	V2X Sidelink Communication Dropping due to synchronization reference source change	815

13.7.3	Interruptions to WAN due to V2X Carrier Aggregation	816

13.7.4	Interruptions to WAN due to NR V2X sidelink communication	816

13.8	Reliability of GNSS signal	816

13.9	Component Carrier Addition and Release Delay for V2X Sidelink Carrier Aggregation	817

13.10	Selection / Reselection of V2X Synchronization Reference Source for V2X Carrier Aggregation	817

Annex A (normative): Test Cases	818

A.1	Purpose of annex	818

A.2	Requirement classification for statistical testing	818

A.2.1	Types of requirements in TS 36.133	818

A.2.1.1	Time and delay requirements on UE higher layer actions	818

A.2.1.2	Measurements of power levels, relative powers and time	818

A.2.1.3	Implementation requirements	819

A.2.1.4	Physical layer timing requirements	819

A.3	RRM test configurations	820

A.3.1	Reference Measurement Channels	820

A.3.1.1	PDSCH	820

A.3.1.1.1	FDD	820

A.3.1.1.2	TDD	825

A.3.1.1.3	FDD for UE category 0	828

A.3.1.1.4	HD-FDD for UE category 0	829

A.3.1.1.5	TDD for UE category 0	830

A.3.1.1.6	Frame Structure 3	831

A.3.1.2	PCFICH/PDCCH/PHICH	832

A.3.1.2.1	FDD	832

A.3.1.2.2	TDD	832

A.3.1.2.3	HD-FDD for UE category 0	833

A.3.1.2.4	FS 3	833

A.3.1.3	MPDCCH Reference Channels for Cat-M1 UEs	833

A.3.1.3.1	FDD in CEModeA	834

A.3.1.3.2	HD-FDD in CEModeA	834

A.3.1.3.3	TDD in CEModeA	835

A.3.1.3.4	FDD in CEModeB	835

A.3.1.3.5	HD-FDD in CEModeB	836

A.3.1.3.6	TDD in CEModeB	836

A.3.1.4	PDSCH Reference Channel for Cat-M1 UEs	837

A.3.1.4.1	FDD in CEModeA	837

A.3.1.4.2	HD-FDD in CEModeA	838

A.3.1.4.3	TDD in CEModeA	839

A.3.1.4.4	FDD in CEModeB	840

A.3.1.4.5	 HD-FDD in CEModeB	841

A.3.1.4.6	TDD in CEModeB	841

A.3.1.5	NPDSCH Reference Channel for UE category NB1	842

A.3.1.5.1	HD-FDD in-band operation	842

A.3.1.5.2	Void	843

A.3.1.5.3	HD-FDD standalone operation	843

A.3.1.5.4	Void	844

A.3.1.5.5	HD-FDD guard band operation	844

A.3.1.5.6	Void	845

A.3.1.5.7	TDD in-band operation	845

A.3.1.5.8	TDD standalone operation	845

A.3.1.5.9	TDD guard band operation	846

A.3.1.6	NPDCCH Reference Channel for UE category NB1	846

A.3.1.6.1	In-band operation	846

A.3.1.6.2	Void	847

A.3.1.6.3	Standalone operation	847

A.3.1.6.4	Void	847

A.3.1.6.5	Guard band operation	847

A.3.1.6.6	Void	848

A.3.2	OFDMA Channel Noise Generator (OCNG)	848

A.3.2.1	OCNG Patterns for FDD	848

A.3.2.1.1	OCNG FDD pattern 1: outer resource blocks allocation in 10 MHz	849

A.3.2.1.2	OCNG FDD pattern 2: full bandwidth allocation in 10 MHz	849

A.3.2.1.3	OCNG FDD pattern 3: outer resource blocks allocation in 1.4 MHz	850

A.3.2.1.4	OCNG FDD pattern 4: full bandwidth allocation in 1.4 MHz	850

A.3.2.1.5	OCNG FDD pattern 5: outer resource blocks allocation in 10 MHz (without MBSFN)	851

A.3.2.1.6	OCNG FDD pattern 6: full bandwidth allocation in 10 MHz (without MBSFN)	852

A.3.2.1.7	OCNG FDD pattern 7: full bandwidth allocation in 1.4 MHz (without MBSFN)	852

A.3.2.1.8	OCNG FDD pattern 8: outer resource blocks allocation in 10 MHz for MBSFN ABS	852

A.3.2.1.9	OCNG FDD pattern 9: full bandwidth allocation in 10 MHz for MBSFN ABS	853

A.3.2.1.10	OCNG FDD pattern 10: outer resource blocks allocation in 10 MHz with user data in every subframe (without MBSFN)	854

A.3.2.1.11	OCNG FDD pattern 11: outer resource blocks allocation in 20 MHz	854

A.3.2.1.12	OCNG FDD pattern 12: full bandwidth allocation in 20 MHz	855

A.3.2.1.13	OCNG FDD pattern 13: outer resource blocks allocation in 20 MHz (without MBSFN)	855

A.3.2.1.14	OCNG FDD pattern 14: full bandwidth allocation in 20 MHz (without MBSFN)	856

A.3.2.1.15	OCNG FDD pattern 15: outer resource blocks allocation in 5 MHz	856

A.3.2.1.16	OCNG FDD pattern 16: full bandwidth allocation in 5 MHz	857

A.3.2.1.17	OCNG FDD pattern 17: outer resource blocks allocation in 20 MHz with user data in every subframe (without MBSFN)	857

A.3.2.1.18	OCNG FDD pattern 18: outer resource blocks allocation in 5 MHz (without MBSFN)	858

A.3.2.1.19	OCNG FDD pattern 19: full bandwidth allocation in 5 MHz (without MBSFN)	858

A.3.2.1.20	OCNG FDD pattern 20: outer resource blocks allocation in 5 MHz with user data in every subframe (without MBSFN)	859

A.3.2.1.21	OCNG FDD pattern 21: Generic resource blocks allocation (without MBSFN)	859

A.3.2.1.22	OCNG FDD pattern 22: Generic resource blocks allocation in 5MHz (without MBSFN)	860

A.3.2.2	OCNG Patterns for TDD	860

A.3.2.2.1	OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	861

A.3.2.2.2	OCNG TDD pattern 2: full bandwidth allocation in 10 MHz	861

A.3.2.2.3	OCNG TDD pattern 3: outer resource blocks allocation in 1.4 MHz	862

A.3.2.2.4	OCNG TDD pattern 4: full bandwidth allocation in 1.4 MHz	862

A.3.2.2.5	OCNG TDD pattern 5: outer resource blocks allocation in 10 MHz for MBSFN ABS	863

A.3.2.2.6	OCNG TDD pattern 6: full bandwidth allocation in 10 MHz for MBSFN ABS	864

A.3.2.2.7	OCNG TDD pattern 7: outer resource blocks allocation in 20 MHz	865

A.3.2.2.8	OCNG TDD pattern 8: full bandwidth allocation in 20 MHz	866

A.3.2.2.9	OCNG TDD pattern 9: outer resource blocks allocation in 5 MHz	866

A.3.2.2.10	OCNG TDD pattern 10: full bandwidth allocation in 5 MHz	867

A.3.2.2.11	OCNG TDD pattern 11: Generic resource blocks allocation (without MBSFN)	867

A.3.2.3	OCNG Patterns for Narrowband IoT	868

A.3.2.3.1	Narrowband IoT OCNG FDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	869

A.3.2.3.2	Narrowband IoT OCNG FDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	870

A.3.2.3.3	Narrowband IoT OCNG FDD pattern 3: standalone NB-IoT	870

A.3.2.3.4	Narrowband IoT OCNG FDD pattern 4: In-band NB-IoT in 5 MHz EUTRAN cell	871

A.3.2.3.5	Narrowband IoT OCNG FDD pattern 5: guard band NB-IoT in 5 MHz EUTRAN cell	872

A.3.2.3.6	Narrowband IoT OCNG TDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	873

A.3.2.3.7	Narrowband IoT OCNG TDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	874

A.3.2.3.8	Narrowband IoT OCNG TDD pattern 3: standalone NB-IoT	875

A.3.2.4	OCNG Patterns for V2X sidelink	876

A.3.2.4.1	V2X sidelink OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	876

A.3.2.4.2	V2X sidelink OCNG TDD pattern 2: outer resource blocks allocation in 10 MHz	877

A.3.3	Reference DRX Configurations	877

A.3.4	ABS Transmission Configurations	878

A.3.4.1	Non-MBSFN ABS Transmission Configurations	878

A.3.4.1.1	Non-MBSFN ABS Transmission, 1x2 antenna with PBCH	878

A.3.4.1.2	Non-MBSFN ABS Transmission, 2x2 antenna without PBCH	878

A.3.4.2	MBSFN ABS Transmission Configurations	879

A.3.4.2.1	MBSFN ABS Transmission, 1x2 antenna	879

A.3.4.2.2	MBSFN ABS Transmission, 2x2 antenna	880

A.3.5	Impact of Reference Sensitivity Degradation with Carrier Aggregation on Test Cases	880

A.3.5.1	Impact of Reference Sensitivity Degradation due to Insertion Loss	880

A.3.6	Carrier Aggregation Test Cases with Different Channel Bandwidth Combinations	881

A.3.6.1	Introduction	881

A.3.7	Test Cases with Different Channel Bandwidths	881

A.3.7.1	Introduction	881

A.3.7.2	Principle of testing	881

A.3.8	Antenna Configuration	881

A.3.8.1	Antenna connection for 4 Rx capable UEs	881

A.3.8.1.1 Introduction	881

A.3.8.1.2 Principle of testing	881

A.3.8.1.2.1 Single carrier tests	881

A.3.8.1.2.2	Carrier aggregation and Dual connectivity tests	882

A.3.8.1.2.3	Antenna connection for bands where 2RX is supported	883

A.3.8.1.2.4	Antenna connection for bands where 4RX is supported	883

A.3.8.2	Antenna connection for 8 Rx capable UEs	883

A.3.8.2.1	Introduction	883

A.3.8.2.2	Principle of testing	883

A.3.8.2.2.1	Single carrier tests	883

A.3.8.2.2.2	Carrier aggregation and Dual connectivity tests	884

A.3.8.2.2.3	Antenna connection for bands where 2RX is supported	884

A.3.8.2.2.4	Antenna connection for bands where 4RX is supported	884

A.3.8.2.2.5	Antenna connection for bands where 8RX is supported	884

A.3.9	Carrier Aggregation Test Cases with Different Duplex Modes	884

A.3.9.1	Introduction	884

A.3.9.2	Principle of testing	884

A.3.10	Carrier Aggregation Test Cases with Different CA Configurations	885

A.3.10.1	Introduction	885

A.3.10.2	Principle of testing	885

A.3.11	Test Cases for Synchronous and Asynchronous Dual Connectivity	885

A.3.11.1	Introduction	885

A.3.11.2	Principle of Testing	885

A.3.12	Proximity-based Services	885

A.3.12.1	Introduction	885

A.3.12.2	Reference DRX configurations for ProSe tests	885

A.3.12.3	Test Cases with Different Channel Bandwidths	886

A.3.12.3.1	Introduction	886

A.3.12.3.2	Principle of testing	886

A.3.12.4	Reference resource pool configurations for ProSe Direct Discovery	886

A.3.12.5	Reference resource pool configurations for ProSe Direct Communication	893

A.3.12.6	Reference Measurement Channels for ProSe Direct Discovery	896

A.3.12.6.1	FDD	896

A.3.12.7	Reference measurement channels for ProSe Direct Communication	896

A.3.12.7.1	FDD	896

A.3.12.8	ProSe Receive Traffic Generator	897

A.3.12.8.1	ProSe Direct Communication Receive Traffic Generator for FDD	897

A.3.12.8.2	ProSe Direct Discovery Receive Traffic Generator for FDD	897

A.3.13	Time Offset between Cells	898

A.3.13.1	Introduction	898

A.3.13.2	Definition	898

A.3.14	Carrier Aggregation under operation with Frame Structure 3 Test Cases with Different Duplex Modes	898

A.3.14.1	Introduction	898

A.3.14.2	Principle of testing	898

A.3.15	Dual connectivity test cases with different combination of duplex mode	898

A.3.15.1	Introduction	898

A.3.15.2	Principle of testing	898

A.3.16	Reference PRACH Configurations	899

A.3.17	Listen before talk model	899

A.3.17.1	Introduction	899

A.3.17.2	Definition	899

A.3.18	Reference NPRACH Configurations	900

A.3.19	Dual connectivity test cases with different bandwidth combinations	901

A.3.19.1	Introduction	901

A.3.19.2	Principle of testing	901

A.3.20	Category M1 UE Test Cases	902

A.3.20.1	Introduction	902

A.3.20.2	Principle of Cat-M1 UE Testing	902

A.3.20.3	Principle of Cat-M1 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	903

A.3.21	V2V Sidelink Communication on Dedicated V2V Carrier	904

A.3.21.1	Introduction	904

A.3.21.2	Reference resource pool configurations for V2V Sidelink Communication	904

A.3.21.3	Reference measurement channels for V2V Sidelink Communication	905

A.3.22	Category 1bis UE Test Cases	906

A.3.22.1	Introduction	906

A.3.22.2	Principle of Category 1bis UE Testing	906

A.3.23	Category NB2 UE Test Cases	910

A.3.23.1	Introduction	910

A.3.23.2	Principle of Category NB2 UE Testing	910

A.3.24	V2X sidelink communication	913

A.3.24.1	Introduction	913

A.3.24.2	Reference resource pool configurations for V2X Sidelink Communication	914

A.3.24.3	Reference measurement channels for V2X Sidelink Communication	918

A.3.25	Category M2 UE Test Cases	919

A.3.25.1	Introduction	919

A.3.25.2	Principle of Cat-M2 UE Testing	919

A.3.25.3	Principle of Cat-M2 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	920

A.3.26	sTTI and processing time reduction test cases with different sTTI/processing time reduction scheme	921

A.3.26.1	Introduction	921

A.3.26.2	Principle of testing	921

A.3.27	LTE INACTIVE Cell Re-selection Test Cases	921

A.3.27.1	Introduction	921

A.3.27.2	Principle of INACTIVE cell re-selection Testing	921

A.4	E-UTRAN RRC_IDLE state	921

A.4.2	Cell Re-Selection	921

A.4.2.1	E-UTRAN FDD – FDD Intra frequency case	921

A.4.2.1.1	Test Purpose and Environment	921

A.4.2.1.2	Test Requirements	923

A.4.2.2	E-UTRAN TDD – TDD Intra frequency case	924

A.4.2.2.1	Test Purpose and Environment	924

A.4.2.2.2	Test Requirements	925

A.4.2.3	E-UTRAN FDD – FDD Inter frequency case	926

A.4.2.3.1	Test Purpose and Environment	926

A.4.2.3.2	Test Requirements	927

A.4.2.4	E-UTRAN FDD – TDD Inter frequency case	928

A.4.2.4.1	Test Purpose and Environment	928

A.4.2.4.2	Test Requirements	929

A.4.2.5	E-UTRAN TDD – FDD Inter frequency case	930

A.4.2.5.1	Test Purpose and Environment	930

A.4.2.5.2	Test Requirements	931

A.4.2.6	E-UTRAN TDD – TDD: Inter frequency case	932

A.4.2.6.1	Test Purpose and Environment	932

A.4.2.6.2	Test Requirements	933

A.4.2.7	E-UTRAN FDD – FDD Inter frequency case in the existence of non-allowed CSG cell	934

A.4.2.7.1	Test Purpose and Environment	934

A.4.2.7.2	Test Requirements	935

A.4.2.8	E-UTRAN TDD – TDD Inter frequency case in the existence of non-allowed CSG cell	936

A.4.2.8.1	Test Purpose and Environment	936

A.4.2.8.2	Test Requirements	937

A.4.2.9	E-UTRAN FDD – FDD Intra frequency case for 5MHz bandwidth	938

A.4.2.9.1	Test Purpose and Environment	938

A.4.2.9.2	Test Requirements	938

A.4.2.10	E-UTRAN FDD – FDD reselection using an increased number of carriers	938

A.4.2.10.1	Test Purpose and Environment	938

A.4.2.10.2	Test Requirements	942

A.4.2.11	E-UTRAN TDD – TDD reselection using an increased number of carriers	942

A.4.2.11.1	Test Purpose and Environment	942

A.4.2.11.2	Test Requirements	946

A.4.2.12	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage	946

A.4.2.12.1	Test Purpose and Environment	946

A.4.2.12.2	Test Requirements	948

A.4.2.13	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage	949

A.4.2.13.1	Test Purpose and Environment	949

A.4.2.13.2	Test Requirements	950

A.4.2.14	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	951

A.4.2.14.1	Test Purpose and Environment	951

A.4.2.14.2	Test Requirements	952

A.4.2.15 E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	953

A.4.2.15.1	Test Purpose and Environment	953

A.4.2.15.2	Test Requirements	954

A.4.2.16 E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	955

A.4.2.16.1	Test Purpose and Environment	955

A.4.2.16.2	Test Requirements	956

A.4.2.17 E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in enhanced coverage	957

A.4.2.17.1	Test Purpose and Environment	957

A.4.2.17.2	Test Requirements	958

A.4.2.18 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	959

A.4.2.18.1	Test Purpose and Environment	959

A.4.2.18.2	Test Requirements	961

A.4.2.19	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	962

A.4.2.19.1	Test Purpose and Environment	962

A.4.2.19.2	Test Requirements	964

A.4.2.20	E-UTRAN FDD – FDD Intra frequency case for UE Category 1bis	965

A.4.2.20.1	Test Purpose and Environment	965

A.4.2.20.2	Test Requirements	966

A.4.2.21	E-UTRAN TDD – TDD Intra frequency case for UE Category 1bis	967

A.4.2.21.1	Test Purpose and Environment	967

A.4.2.21.2	Test Requirements	968

A.4.2.22	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	969

A.4.2.22.1	Test Purpose and Environment	969

A.4.2.22.2	Test Requirements	970

A.4.2.23	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	971

A.4.2.23.1	Test Purpose and Environment	971

A.4.2.23.2	Test Requirements	972

A.4.2.24	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	973

A.4.2.24.1	Test Purpose and Environment	973

A.4.2.24.2	Test Requirements	976

A.4.2.25	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	977

A.4.2.25.1	Test Purpose and Environment	977

A.4.2.25.2	Test Requirements	978

A.4.2.26	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage	979

A.4.2.26.1	Test Purpose and Environment	979

A.4.2.26.2	Test Requirements	980

A.4.2.27	E-UTRAN TDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	981

A.4.2.27.1	Test Purpose and Environment	981

A.4.2.27.2	Test Requirements	982

A.4.2.28	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	983

A.4.2.28.1	Test Purpose and Environment	983

A.4.2.28.2	Test Requirements	984

A.4.2.29	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	985

A.4.2.29.1	Test Purpose and Environment	985

A.4.2.29.2	Test Requirements	986

A.4.2.30	E-UTRAN TDD Inter frequency case for Cat-M1 UE in enhanced coverage	987

A.4.2.30.1	Test Purpose and Environment	987

A.4.2.30.2	Test Requirements	988

A.4.2.31	E-UTRAN FDD – FDD Inter frequency case for UE Category 1bis	989

A.4.2.31.1	Test Purpose and Environment	989

A.4.2.31.2	Test Requirements	990

A.4.2.32	E-UTRAN FDD – TDD Inter frequency case for UE Category 1bis	991

A.4.2.32.1	Test Purpose and Environment	991

A.4.2.32.2	Test Requirements	992

A.4.2.33	E-UTRAN TDD – FDD Inter frequency case for UE Category 1bis	993

A.4.2.33.1	Test Purpose and Environment	993

A.4.2.33.2	Test Requirements	994

A.4.2.34	E-UTRAN TDD – TDD: Inter frequency case for UE Category 1bis	995

A.4.2.34.1	Test Purpose and Environment	995

A.4.2.34.2	Test Requirements	996

A.4.2.35	E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	997

A.4.2.35.1	Test Purpose and Environment	997

A.4.2.35.2	Test Requirements	999

A.4.2.36	E-UTRAN TDD – TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	1000

A.4.2.36.1	Test Purpose and Environment	1000

A.4.2.36.2	Test Requirements	1002

A.4.2.37	E-UTRAN TDD – TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	1003

A.4.2.37.1	Test Purpose and Environment	1003

A.4.2.37.2	Test Requirements	1005

A.4.2.38	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation	1006

A.4.2.38.1	Test Purpose and Environment	1006

A.4.2.38.2	Test Requirements	1008

A.4.2.39	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhMeasFlag2-r16	1009

A.4.2.39.1	Test Purpose and Environment	1009

A.4.2.39.2	Test Requirements	1010

A.4.2.40	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhMeasFlag2-r16	1011

A.4.2.40.1	Test Purpose and Environment	1011

A.4.2.40.2	Test Requirements	1012

A.4.2.41 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with UE specific DRX	1013

A.4.2.41.1	Test Purpose and Environment	1013

A.4.2.41.2	Test Requirements	1016

A.4.2.42.2	Test Requirements	1020

A.4.2.43.2	Test Requirements	1023

A.4.2.44.2	Test Requirements	1027

A.4.2.45	E-UTRAN TDD – TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1027

A.4.2.45.1	Test Purpose and Environment	1027

A.4.2.45.2	Test Requirements	1030

A.4.2.46.2	Test Requirements	1034

A.4.2.47	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation with UE specific DRX	1034

A.4.2.47.1	Test Purpose and Environment	1034

A.4.2.47.2	Test Requirements	1038

A.4.2.48	E-UTRAN FD-FDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1038

A.4.2.48.1	Test Purpose and Environment	1038

A.4.2.48.2	Test Requirements	1040

A.4.2.49	E-UTRAN HD-FDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1041

A.4.2.49.1	Test Purpose and Environment	1041

A.4.2.49.2	Test Requirements	1043

A.4.2.50	E-UTRAN TDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1044

A.4.2.50.1	Test Purpose and Environment	1044

A.4.2.50.2	Test Requirements	1046

A.4.2.51 E-UTRAN FD-FDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1047

A.4.2.51.1	Test Purpose and Environment	1047

A.4.2.51.2	Test Requirements	1049

A.4.2.52 E-UTRAN HD-FDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1050

A.4.2.52.1	Test Purpose and Environment	1050

A.4.2.52.2	Test Requirements	1052

A.4.2.53 E-UTRAN TDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1053

A.4.2.53.1	Test Purpose and Environment	1053

A.4.2.53.2	Test Requirements	1054

A.4.2.54	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	1055

A.4.2.54.1	Test Purpose and Environment	1055

A.4.2.54.2	Test Requirements	1057

A.4.2.55	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	1058

A.4.2.55.1	Test Purpose and Environment	1058

A.4.2.55.2	Test Requirements	1060

A.4.2.56	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	1061

A.4.2.56.1	Test Purpose and Environment	1061

A.4.2.56.2	Test Requirements	1062

A.4.3	E-UTRAN to UTRAN Cell Re-Selection	1063

A.4.3.1	E-UTRAN FDD – UTRAN FDD:	1063

A.4.3.1.1	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of higher priority	1063

A.4.3.1.1.1	Test Purpose and Environment	1063

A.4.3.1.1.2	Test Requirements	1065

A.4.3.1.2	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority	1065

A.4.3.1.2.1	Test Purpose and Environment	1065

A.4.3.1.2.2	Test Requirements	1068

A.4.3.1.3	EUTRA FDD-UTRA FDD cell reselection in fading propagation conditions: UTRA FDD is of lower priority	1068

A.4.3.1.3.1	Test Purpose and Environment	1068

A.4.3.1.3.2	Test Requirements	1071

A.4.3.1.4	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority for 5MHz bandwidth	1071

A.4.3.1.4.1	Test Purpose and Environment	1071

A.4.3.1.4.2	Test Requirements	1072

A.4.3.1.5	Idle mode FDD to UTRA FDD interRAT reselection	1072

A.4.3.1.5.1	Test Purpose and Environment	1072

A.4.3.1.5.2	Test Requirements	1075

A.4.3.2	E-UTRAN FDD – UTRAN TDD:	1076

A.4.3.2.1	Test Purpose and Environment	1076

A.4.3.2.1.1	Void	1076

A.4.3.2.1.2	1.28Mcps TDD option	1076

A.4.3.2.1.3	Void	1078

A.4.3.2.2	Test Requirements	1078

A.4.3.2.2.1	1.28Mcps TDD option	1078

A.4.3.2A	E-UTRA FDD to UTRA TDD cell re-selection for IncMon	1078

A.4.3.2A.1	Test Purpose and Environment	1078

A.4.3.2A.2	Test Requirements	1082

A.4.3.3	E-UTRAN TDD – UTRAN FDD:	1082

A.4.3.3.1	Test Purpose and Environment	1082

A.4.3.3.2	Test Requirements	1085

A.4.3.3A	Idle mode TDD to UTRA FDD interRAT reselection	1085

A.4.3.3A.1	Test Purpose and Environment	1085

A.4.3.3A.2	Test Requirements	1090

A.4.3.4	E-UTRAN TDD – UTRAN TDD:	1091

A.4.3.4.1	E-UTRA to UTRA TDD cell re-selection: UTRA is of higher priority	1091

A.4.3.4.1.1	Test Purpose and Environment	1091

A.4.3.4.1.2	Test Requirements	1093

A.4.3.4.2	E-UTRA to UTRA TDD cell re-selection: UTRA is of lower priority	1093

A.4.3.4.2.1	Test Purpose and Environment	1093

A.4.3.4.2.2	Test Requirements	1095

A.4.3.4.3	EUTRA TDD-UTRA TDD cell reselection in fading propagation conditions: UTRA TDD is of lower priority	1095

A.4.3.4.3.1	Test Purpose and Environment	1095

A.4.3.4.3.2	Test Requirements	1098

A.4.3.4.4	E-UTRA TDD to UTRA TDD cell re-selection for IncMon	1098

A.4.3.4.4.1	Test Purpose and Environment	1098

A.4.3.4.4.2	Test Requirements	1102

A.4.4	E-UTRAN to GSM Cell Re-Selection	1102

A.4.4.1	E-UTRAN FDD – GSM:	1102

A.4.4.1.1	Test Purpose and Environment	1102

A.4.4.1.2	Test Requirements	1104

A.4.4.2	E-UTRAN TDD – GSM:	1104

A.4.4.2.1	Test Purpose and Environment	1104

A.4.4.2.2	Test Requirements	1106

A.4.5	E-UTRAN to HRPD Cell Re-Selection	1107

A.4.5.1	E-UTRAN FDD – HRPD	1107

A.4.5.1.1	E-UTRAN FDD – HRPD Cell Reselection: HRPD is of Lower Priority	1107

A.4.5.1.1.1	Test Purpose and Environment	1107

A.4.5.1.1.2	Test Requirements	1109

A.4.5.2	E-UTRAN TDD – HRPD	1109

A.4.5.2.1	E-UTRAN TDD – HRPD Cell Reselection: HRPD is of Lower Priority	1109

A.4.5.2.1.1	Test Purpose and Environment	1109

A.4.5.2.1.2	Test Requirements	1112

A.4.6	E-UTRAN to cdma2000 1X Cell Re-Selection	1112

A.4.6.1	E-UTRAN FDD – cdma2000 1X	1112

A.4.6.1.1	E-UTRAN FDD – cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	1112

A.4.6.1.1.1	Test Purpose and Environment	1112

A.4.6.1.1.2	Test Requirements	1115

A.4.6.2	E-UTRAN TDD – cdma2000 1X	1115

A.4.6.2.1	E-UTRAN TDD –cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	1115

A.4.6.2.1.1	Test Purpose and Environment	1115

A.4.6.2.1.2	Test Requirements	1118

A.4.7	Idle State Positioning Measurement for UE category NB1	1118

A.4.7.1	HD – FDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	1118

A.4.7.1.1	Test Purpose and Environment	1118

A.4.7.1.2	Test Requirements	1123

A.4.7.2	HD – FDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	1124

A.4.7.2.1	Test Purpose and Environment	1124

A.4.7.4.2	Test Requirements	1128

A.4.7.3	TDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	1129

A.4.7.3.1	Test Purpose and Environment	1129

A.4.7.3.2	Test Requirements	1133

A.4.7.4	TDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	1134

A.4.7.4.1	Test Purpose and Environment	1134

A.4.7.4.2	Test Requirements	1138

A.5	E-UTRAN RRC CONNECTED Mode Mobility	1139

A.5.1	E-UTRAN Handover	1139

A.5.1.1	E-UTRAN FDD - FDD Intra frequency handover	1139

A.5.1.1.1	Test Purpose and Environment	1139

A.5.1.1.2	Test Requirements	1141

A.5.1.2	E-UTRAN TDD - TDD Intra frequency handover	1141

A.5.1.2.1	Test Purpose and Environment	1141

A.5.1.2.2	Test Requirements	1143

A.5.1.3	E-UTRAN FDD – FDD Inter frequency handover	1143

A.5.1.3.1	Test Purpose and Environment	1143

A.5.1.3.2	Test Requirements	1145

A.5.1.4	E-UTRAN TDD – TDD Inter frequency handover	1145

A.5.1.4.1	Test Purpose and Environment	1145

A.5.1.4.2	Test Requirements	1147

A.5.1.5	E-UTRAN FDD – FDD Inter frequency handover: unknown target cell	1147

A.5.1.5.1	Test Purpose and Environment	1147

A.5.1.5.2	Test Requirements	1149

A.5.1.6 	E-UTRAN TDD – TDD Inter frequency handover; unknown Target Cell	1149

A.5.1.6.1	Test Purpose and Environment	1149

A.5.1.6.2	Test Requirements	1151

A.5.1.7	E-UTRAN FDD – TDD Inter frequency handover	1151

A.5.1.7.1	Test Purpose and Environment	1151

A.5.1.7.2	Test Requirements	1154

A.5.1.8	E-UTRAN TDD – FDD Inter frequency handover	1154

A.5.1.8.1	Test Purpose and Environment	1154

A.5.1.8.2	Test Requirements	1157

A.5.1.9	E-UTRAN FDD - FDD Intra frequency handover for 5MHz bandwidth	1157

A.5.1.9.1	Test Purpose and Environment	1157

A.5.1.9.2	Test Requirements	1158

A.5.1.10	E-UTRAN FDD - FDD Intra frequency handover for UE category 0	1158

A.5.1.10.1	Test Purpose and Environment	1158

A.5.1.10.2	Test Requirements	1160

A.5.1.11	E-UTRAN HD - FDD Intra frequency handover for UE category 0	1160

A.5.1.11.1	Test Purpose and Environment	1160

A.5.1.11.2	Test Requirements	1162

A.5.1.12	E-UTRAN TDD - TDD Intra frequency handover for UE category 0	1162

A.5.1.12.1	Test Purpose and Environment	1162

A.5.1.12.2	Test Requirements	1164

A.5.1.13	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1164

A.5.1.13.1	Test Purpose and Environment	1164

A.5.1.13.2	Test Requirements	1166

A.5.1.14	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1167

A.5.1.14.1	Test Purpose and Environment	1167

A.5.1.14.2	Test Requirements	1168

A.5.1.15	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA	1169

A.5.1.15.1	Test Purpose and Environment	1169

A.5.1.15.2	Test Requirements	1170

A.5.1.16	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1171

A.5.1.16.1	Test Purpose and Environment	1171

A.5.1.16.2	Test Requirements	1172

A.5.1.17	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1173

A.5.1.17.1	Test Purpose and Environment	1173

A.5.1.17.2	Test Requirements	1174

A.5.1.18	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB	1175

A.5.1.18.1	Test Purpose and Environment	1175

A.5.1.18.2	Test Requirements	1176

A.5.1.19	E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis	1177

A.5.1.19.1	Test Purpose and Environment	1177

A.5.1.19.2	Test Requirements	1178

A.5.1.20	E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis	1179

A.5.1.20.1	Test Purpose and Environment	1179

A.5.1.20.2	Test Requirements	1180

A.5.1.21	E-UTRAN FDD - FDD Intra frequency RACH-less handover	1181

A.5.1.21.1	Test Purpose and Environment	1181

A.5.1.21.2	Test Requirements	1182

A.5.1.22	E-UTRAN TDD - TDD Intra frequency RACH-less handover	1182

A.5.1.22.1	Test Purpose and Environment	1182

A.5.1.22.2	Test Requirements	1184

A.5.1.23	E-UTRAN FDD – FDD Inter frequency RACH-less handover	1184

A.5.1.23.1	Test Purpose and Environment	1184

A.5.1.23.2	Test Requirements	1186

A.5.1.24	E-UTRAN TDD – TDD Inter frequency RACH-less handover	1186

A.5.1.24.1	Test Purpose and Environment	1186

A.5.1.24.2	Test Requirements	1188

A.5.1.25	E-UTRAN FDD - FDD Intra frequency make-before-break handover	1188

A.5.1.25.1	Test Purpose and Environment	1188

A.5.1.25.2	Test Requirements	1190

A.5.1.26	E-UTRAN TDD - TDD Intra frequency make-before-break handover	1191

A.5.1.26.1	Test Purpose and Environment	1191

A.5.1.26.2	Test Requirements	1192

A.5.1.27	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeA	1193

A.5.1.27.1	Test Purpose and Environment	1193

A.5.1.27.2	Test Requirements	1194

A.5.1.28	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeA	1195

A.5.1.28.1	Test Purpose and Environment	1195

A.5.1.28.2	Test Requirements	1196

A.5.1.29	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeA	1197

A.5.1.29.1	Test Purpose and Environment	1197

A.5.1.29.2	Test Requirements	1198

A.5.1.30	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeB	1199

A.5.1.30.1	Test Purpose and Environment	1199

A.5.1.30.2	Test Requirements	1200

A.5.1.31	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeB	1201

A.5.1.31.1	Test Purpose and Environment	1201

A.5.1.31.2	Test Requirements	1202

A.5.1.32	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeB	1203

A.5.1.32.1	Test Purpose and Environment	1203

A.5.1.32.2	Test Requirements	1204

A.5.1.33	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1205

A.5.1.33.1	Test Purpose and Environment	1205

A.5.1.13.2	Test Requirements	1206

A.5.1.34	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1207

A.5.1.34.1	Test Purpose and Environment	1207

A.5.1.34.2	Test Requirements	1208

A.5.1.35	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1209

A.5.1.35.1	Test Purpose and Environment	1209

A.5.1.35.2	Test Requirements	1210

A.5.1.36	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1211

A.5.1.36.1	Test Purpose and Environment	1211

A.5.1.36.2	Test Requirements	1212

A.5.1.37	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1213

A.5.1.37.1	Test Purpose and Environment	1213

A.5.1.37.2	Test Requirements	1214

A.5.1.38	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1215

A.5.1.38.1	Test Purpose and Environment	1215

A.5.1.38.2	Test Requirements	1216

A.5.1.39	E-UTRAN FDD - FDD Intra frequency handover with direct SCell activation	1217

A.5.1.39.1	Test Purpose and Environment	1217

A.5.1.39.2	Test Requirements	1218

A.5.1.40	E-UTRAN TDD - TDD Intra frequency handover with direct SCell activation	1219

A.5.1.40.1	Test Purpose and Environment	1219

A.5.1.40.2	Test Requirements	1220

A.5.1.41	E-UTRAN FDD – FDD Intra-band Inter-frequency sync DAPS handover	1221

A.5.1.41.1	Test Purpose and Environment	1221

A.5.1.41.2	Test Requirements	1223

A.5.1.42	E-UTRAN FDD – FDD Intra-band Inter-frequency async DAPS handover	1224

A.5.1.42.1	Test Purpose and Environment	1224

A.5.1.42.2	Test Requirements	1225

A.5.1.43	E-UTRAN FDD – FDD Inter-band Inter-frequency sync DAPS handover	1226

A.5.1.43.1	Test Purpose and Environment	1226

A.5.1.43.2	Test Requirements	1227

A.5.1.44	E-UTRAN FDD – FDD Inter-band Inter-frequency async DAPS handover	1228

A.5.1.44.1	Test Purpose and Environment	1228

A.5.1.44.2	Test Requirements	1229

A.5.2	E-UTRAN Handover to other RATs	1248

A.5.2.1	E-UTRAN FDD – UTRAN FDD Handover	1248

A.5.2.1.1	Test Purpose and Environment	1248

A.5.2.1.2	Test Requirements	1249

A.5.2.2	E-UTRAN TDD - UTRAN FDD Handover	1250

A.5.2.2.1	Test Purpose and Environment	1250

A.5.2.2.2	Test Requirements	1253

A.5.2.3 E-UTRAN FDD- GSM Handover	1253

A.5.2.3.1	Test Purpose and Environment	1253

A.5.2.3.2	Test Requirements	1254

A.5.2.4	E-UTRAN TDD - UTRAN TDD Handover	1255

A.5.2.4.1	Test Purpose and Environment	1255

A.5.2.4.1.1	Void	1255

A.5.2.4.1.2	1.28 Mcps TDD option	1255

A.5.2.4.1.3	Void	1257

A.5.2.4.2	Test Requirements	1257

A.5.2.4.2.1	Void	1257

A.5.2.4.2.2	1.28 Mcps TDD option	1257

A.5.2.4.2.3	Void	1257

A.5.2.5	E-UTRAN FDD – UTRAN TDD Handover	1257

A.5.2.5.1	Test Purpose and Environment	1257

A.5.2.5.1.3	Void	1260

A.5.2.5.2	Test Requirements	1260

A.5.2.5.2.1	Void	1260

A.5.2.5.2.2	1.28 Mcps TDD option	1260

A.5.2.5.2.3	Void	1260

A.5.2.6	E-UTRAN TDD - GSM Handover	1260

A.5.2.6.1	Test Purpose and Environment	1260

A.5.2.6.2	Test Requirements	1262

A.5.2.7	E-UTRAN FDD – UTRAN FDD Handover; Unknown Target Cell	1263

A.5.2.7.1	Test Purpose and Environment	1263

A.5.2.7.2	Test Requirements	1265

A.5.2.8	E-UTRAN FDD - GSM Handover; Unknown Target Cell	1265

A.5.2.8.1	Test Purpose and Environment	1265

A.5.2.8.2	Test Requirements	1266

A.5.2.9	E-UTRAN TDD - GSM Handover; Unknown Target Cell	1267

A.5.2.9.1	Test Purpose and Environment	1267

A.5.2.9.2	Test Requirements	1268

A.5.2.10	E-UTRAN TDD to UTRAN TDD handover: unknown target cell	1269

A.5.2.10.1	Test Purpose and Environment	1269

A.5.2.10.2	Test Requirements	1271

A.5.2.10A	E-UTRAN FDD – UTRAN FDD Multicarrier Handover with two target cells	1271

A.5.2.10A.1	Test Purpose and Environment	1271

A.5.2.10A.2	Test Requirements	1274

A.5.2.10B	E-UTRAN TDD – UTRAN FDD Multicarrier Handover with two target cells	1274

A.5.2.10B.1	Test Purpose and Environment	1274

A.5.2.10B.2	Test Requirements	1277

A.5.2.11	E-UTRAN FDD – UTRAN FDD Handover for 5MHz Bandwidth	1277

A.5.2.11.1	Test Purpose and Environment	1277

A.5.2.11.2	Test Requirements	1277

A.5.3	E-UTRAN Handover to Non-3GPP RATs	1278

A.5.3.1	E-UTRAN FDD – HRPD Handover	1278

A.5.3.1.1	Test Purpose and Environment	1278

A.5.3.1.2	Test Requirements	1280

A.5.3.2	E-UTRAN FDD – cdma2000 1X Handover	1280

A.5.3.2.1	Test Purpose and Environment	1280

A.5.3.2.2	Test Requirements	1283

A.5.3.3	E-UTRAN FDD – HRPD Handover; Unknown Target Cell	1283

A.5.3.3.1	Test Purpose and Environment	1283

A.5.3.3.2	Test Requirements	1285

A.5.3.4	E-UTRAN FDD – cdma2000 1X Handover; Unknown Target cell	1286

A.5.3.4.1	Test Purpose and Environment	1286

A.5.3.4.2	Test Requirements	1287

A.5.3.5	E-UTRAN TDD – HRPD Handover	1288

A.5.3.5.1	Test Purpose and Environment	1288

A.5.3.5.2	Test Requirements	1290

A.5.3.6	E-UTRAN TDD – cdma2000 1X Handover	1291

A.5.3.6.1	Test Purpose and Environment	1291

A.5.3.6.2	Test Requirements	1294

A.6	RRC Connection Control	1294

A.6.1	RRC Re-establishment	1294

A.6.1.1	E-UTRAN FDD Intra-frequency RRC Re-establishment	1294

A.6.1.1.1	Test Purpose and Environment	1294

A.6.1.1.2	Test Requirements	1295

A.6.1.2	E-UTRAN FDD Inter-frequency RRC Re-establishment	1296

A.6.1.2.1	Test Purpose and Environment	1296

A.6.1.2.2	Test Requirements	1297

A.6.1.3	E-UTRAN TDD Intra-frequency RRC Re-establishment	1298

A.6.1.3.1	Test Purpose and Environment	1298

A.6.1.3.2	Test Requirements	1299

A.6.1.4	E-UTRAN TDD Inter-frequency RRC Re-establishment	1300

A.6.1.4.1	Test Purpose and Environment	1300

A.6.1.4.2	Test Requirements	1301

A.6.1.5	E-UTRAN FDD Intra-frequency RRC Re-establishment for 5MHz bandwidth	1302

A.6.1.5.1	Test Purpose and Environment	1302

A.6.1.5.2	Test Requirements	1302

A.6.1.6	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for UE category 0	1302

A.6.1.6.1	Test Purpose and Environment	1302

A.6.1.6.2	Test Requirements	1304

A.6.1.7	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for UE category 0	1305

A.6.1.7.1	Test Purpose and Environment	1305

A.6.1.7.2	Test Requirements	1306

A.6.1.8	E-UTRAN TDD Intra-frequency RRC Re-establishment for UE category 0	1307

A.6.1.8.1	Test Purpose and Environment	1307

A.6.1.8.2	Test Requirements	1308

A.6.1.9	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1309

A.6.1.9.1	Test Purpose and Environment	1309

A.6.1.9.2	Test Requirements	1310

A.6.1.10	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1311

A.6.1.10.1	Test Purpose and Environment	1311

A.6.1.10.2	Test Requirements	1312

A.6.1.11	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1313

A.6.1.11.1	Test Purpose and Environment	1313

A.6.1.11.2	Test Requirements	1314

A.6.1.12	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1315

A.6.1.12.1	Test Purpose and Environment	1315

A.6.1.12.2	Test Requirements	1316

A.6.1.13	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1317

A.6.1.13.1	Test Purpose and Environment	1317

A.6.1.13.2	Test Requirements	1318

A.6.1.14	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1319

A.6.1.14.1	Test Purpose and Environment	1319

A.6.1.14.2	Test Requirements	1320

A.6.1.15	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1321

A.6.1.15.1	Test Purpose and Environment	1321

A.6.1.15.2	Test Requirements	1323

A.6.1.16	HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1324

A.6.1.16.1	Test Purpose and Environment	1324

A.6.1.16.2	Test Requirements	1326

A.6.1.17	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1327

A.6.1.17.1	Test Purpose and Environment	1327

A.6.1.17.2	Test Requirements	1328

A.6.1.18	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1329

A.6.1.18.1	Test Purpose and Environment	1329

A.6.1.18.2	Test Requirements	1330

A.6.1.19	E-UTRAN TDD-TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1331

A.6.1.19.1	Test Purpose and Environment	1331

A.6.1.19.2	Test Requirements	1332

A.6.1.20	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1333

A.6.1.20.1	Test Purpose and Environment	1333

A.6.1.20.2	Test Requirements	1334

A.6.1.21	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1335

A.6.1.21.1	Test Purpose and Environment	1335

A.6.1.21.2	Test Requirements	1336

A.6.1.22	E-UTRAN TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1337

A.6.1.22.1	Test Purpose and Environment	1337

A.6.1.22.2	Test Requirements	1338

A.6.1.23	E-UTRAN TDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1339

A.6.1.23.1	Test Purpose and Environment	1339

A.6.1.23.2	Test Requirements	1341

A.6.1.24	E-UTRAN TDD - TDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1342

A.6.1.24.1	Test Purpose and Environment	1342

A.6.1.24.2	Test Requirements	1344

A.6.2	Random Access	1345

A.6.2.1	E-UTRAN FDD – Contention Based Random Access Test	1345

A.6.2.1.1	Test Purpose and Environment	1345

A.6.2.1.2	Test Requirements	1346

A.6.2.1.2.1	Random Access Response Reception	1346

A.6.2.1.2.2	No Random Access Response Reception	1346

A.6.2.1.2.3	Receiving a NACK on msg3	1346

A.6.2.1.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1347

A.6.2.1.2.5	Reception of a Correct Message over Temporary C-RNTI	1347

A.6.2.1.2.6	Contention Resolution Timer expiry	1347

A.6.2.2	E-UTRAN FDD – Non-Contention Based Random Access Test	1347

A.6.2.2.1	Test Purpose and Environment	1347

A.6.2.2.2	Test Requirements	1348

A.6.2.2.2.1	Random Access Response Reception	1349

A.6.2.2.2.2	No Random Access Response Reception	1349

A.6.2.3	E-UTRAN TDD – Contention Based Random Access Test	1349

A.6.2.3.1	Test Purpose and Environment	1349

A.6.2.3.2	Test Requirements	1351

A.6.2.3.2.1	Random Access Response Reception	1351

A.6.2.3.2.2	No Random Access Response reception	1351

A.6.2.3.2.3	Receiving a NACK on msg3	1351

A.6.2.3.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1352

A.6.2.3.2.5	Reception of a Correct Message over Temporary C-RNTI	1352

A.6.2.3.2.6	Contention Resolution Timer expiry	1352

A.6.2.4	E-UTRAN TDD – Non-Contention Based Random Access Test	1352

A.6.2.4.1	Test Purpose and Environment	1352

A.6.2.4.2	Test Requirements	1354

A.6.2.4.2.1	Random Access Response Reception	1354

A.6.2.4.2.2	No Random Access Response Reception	1354

A.6.2.5	E-UTRAN FDD – Contention Based Random Access Test for 5MHz bandwidth	1354

A.6.2.5.1	Test Purpose and Environment	1354

A.6.2.5.2	Test Requirements	1355

A.6.2.6	E-UTRAN FDD – Non-contention Based Random Access Test for 5MHz bandwidth	1355

A.6.2.6.1	Test Purpose and Environment	1355

A.6.2.6.2	Test Requirements	1355

A.6.2.7	E-UTRAN FDD – Non-Contention Based Random Access Test For SCell	1356

A.6.2.7.1	Test Purpose and Environment	1356

A.6.2.7.2	Test Requirements	1358

A.6.2.7.2.1	Random Access Response Reception	1358

A.6.2.7.2.2	No Random Access Response Reception	1358

A.6.2.7.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1358

A.6.2.8	E-UTRAN TDD – Non-Contention Based Random Access Test For SCell	1359

A.6.2.8.1	Test Purpose and Environment	1359

A.6.2.8.2	Test Requirements	1361

A.6.2.8.2.1	Random Access Response Reception	1361

A.6.2.8.2.2	No Random Access Response Reception	1361

A.6.2.8.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1361

A.6.2.9	3DL/3UL TDD CA Non-Contention Based Random Access Test for 2 SCells	1362

A.6.2.9.1	Test Purpose and Environment	1362

A.6.2.9.2	Test Requirements	1365

A.6.2.9.2.1	Random Access Response Reception	1365

A.6.2.9.2.2	No Random Access Response Reception	1366

A.6.2.9.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1366

A.6.2.10	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1367

A.6.2.10.1	Test Purpose and Environment	1367

A.6.2.10.2	Test Requirements	1369

A.6.2.10.2.1	Random Access Response Reception	1369

A.6.2.10.2.2	No Random Access Response Reception	1370

A.6.2.10.2.3	Receiving a NACK on msg3	1370

A.6.2.10.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1370

A.6.2.10.2.5	Reception of a Correct Message over Temporary C-RNTI	1370

A.6.2.10.2.6	Contention Resolution Timer expiry	1370

A.6.2.10.2.7	PRACH Resource Selection	1371

A.6.2.11	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1371

A.6.2.11.1	Test Purpose and Environment	1371

A.6.2.11.2	Test Requirements	1373

A.6.2.11.2.1	Random Access Response Reception	1373

A.6.2.11.2.2	No Random Access Response Reception	1374

A.6.2.11.2.3	Receiving a NACK on msg3	1374

A.6.2.11.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1374

A.6.2.11.2.5	Reception of a Correct Message over Temporary C-RNTI	1374

A.6.2.11.2.6	Contention Resolution Timer expiry	1374

A.6.2.11.2.7	PRACH Resource Selection	1375

A.6.2.12	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1375

A.6.2.12.1	Test Purpose and Environment	1375

A.6.2.12.2	Test Requirements	1377

A.6.2.12.2.1	Random Access Response Reception	1377

A.6.2.12.2.2	No Random Access Response Reception	1378

A.6.2.12.2.3	Receiving a NACK on msg3	1378

A.6.2.12.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1378

A.6.2.12.2.5	Reception of a Correct Message over Temporary C-RNTI	1378

A.6.2.12.2.6	Contention Resolution Timer expiry	1378

A.6.2.12.2.7	PRACH Resource Selection	1379

A.6.2.13	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1379

A.6.2.13.1	Test Purpose and Environment	1379

A.6.2.13.2	Test Requirements	1381

A.6.2.13.2.1	Random Access Response Reception	1381

A.6.2.13.2.2	No Random Access Response Reception	1382

A.6.2.13.2.3	Receiving a NACK on msg3	1382

A.6.2.13.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1382

A.6.2.13.2.5	Reception of a Correct Message over Temporary C-RNTI	1382

A.6.2.13.2.6	Contention Resolution Timer expiry	1382

A.6.2.13.2.7	PRACH Resource Selection	1383

A.6.2.14	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1383

A.6.2.14.1	Test Purpose and Environment	1383

A.6.2.14.2	Test Requirements	1385

A.6.2.14.2.1	Random Access Response Reception	1385

A.6.2.14.2.2	No Random Access Response Reception	1386

A.6.2.14.2.3	Receiving a NACK on msg3	1386

A.6.2.14.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1386

A.6.2.14.2.5	Reception of a Correct Message over Temporary C-RNTI	1386

A.6.2.14.2.6	Contention Resolution Timer expiry	1386

A.6.2.14.2.7	PRACH Resource Selection	1387

A.6.2.15	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1387

A.6.2.15.1	Test Purpose and Environment	1387

A.6.2.15.2	Test Requirements	1389

A.6.2.15.2.1	Random Access Response Reception	1389

A.6.2.15.2.2	No Random Access Response Reception	1390

A.6.2.15.2.3	Receiving a NACK on msg3	1390

A.6.2.15.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1390

A.6.2.15.2.5	Reception of a Correct Message over Temporary C-RNTI	1390

A.6.2.15.2.6	Contention Resolution Timer expiry	1390

A.6.2.15.2.7	PRACH Resource Selection	1391

A.6.2.16	Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1391

A.6.2.16.1	Test Purpose and Environment	1391

A.6.2.16.2	Test Requirements	1394

A.6.2.16.2.1	Random Access Response Reception	1394

A.6.2.16.2.2	No Random Access Response Reception	1395

A.6.2.16.2.3	Receiving a NACK on msg3	1395

A.6.2.16.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1395

A.6.2.16.2.5	Reception of a Correct Message over Temporary C-RNTI	1395

A.6.2.16.2.6	Contention Resolution Timer expiry	1395

A.6.2.16.2.7	NPRACH Resource Selection	1395

A.6.2.17	Contention Based Random Access Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1396

A.6.2.17.1	Test Purpose and Environment	1396

A.6.2.17.2	Test Requirements	1399

A.6.2.17.2.1	Random Access Response Reception	1399

A.6.2.17.2.2	No Random Access Response Reception	1400

A.6.2.17.2.3	Receiving a NACK on msg3	1400

A.6.2.17.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1400

A.6.2.17.2.5	Reception of a Correct Message over Temporary C-RNTI	1400

A.6.2.17.2.6	Contention Resolution Timer expiry	1400

A.6.2.17.2.7	NPRACH Resource Selection	1400

A.6.2.18	Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1401

A.6.2.18.1	Test Purpose and Environment	1401

A.6.2.18.2	Test Requirements	1404

A.6.2.18.2.1	Random Access Response Reception	1404

A.6.2.18.2.2	No Random Access Response Reception	1405

A.6.2.18.2.3	Receiving a NACK on msg3	1405

A.6.2.18.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1405

A.6.2.18.2.5	Reception of a Correct Message over Temporary C-RNTI	1405

A.6.2.18.2.6	Contention Resolution Timer expiry	1405

A.6.2.18.2.7	NPRACH Resource Selection	1405

A.6.2.19	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1406

A.6.2.19.1	Test Purpose and Environment	1406

A.6.2.19.2	Test Requirements	1408

A.6.2.19.2.1	Random Access Response Reception	1408

A.6.2.19.2.2	No Random Access Response Reception	1409

A.6.2.19.2.3	Receiving a NACK on msg3	1409

A.6.2.19.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1409

A.6.2.19.2.5	Reception of a Correct Message over Temporary C-RNTI	1409

A.6.2.19.2.6	Contention Resolution Timer expiry	1409

A.6.2.19.2.7	NPRACH Resource Selection	1409

A.6.2.20	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in enhanced coverage	1410

A.6.2.20.1	Test Purpose and Environment	1410

A.6.2.20.2	Test Requirements	1412

A.6.2.20.2.1	Random Access Response Reception	1412

A.6.2.20.2.2	No Random Access Response Reception	1413

A.6.2.20.2.3	Receiving a NACK on msg3	1413

A.6.2.20.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1413

A.6.2.20.2.5	Reception of a Correct Message over Temporary C-RNTI	1413

A.6.2.20.2.6	Contention Resolution Timer expiry	1413

A.6.2.20.2.7	NPRACH Resource Selection	1413

A.6.2.21	TDD Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1414

A.6.2.21.1	Test Purpose and Environment	1414

A.6.2.21.2	Test Requirements	1416

A.6.2.21.2.1	Random Access Response Reception	1416

A.6.2.21.2.2	No Random Access Response Reception	1417

A.6.2.21.2.3	Receiving a NACK on msg3	1417

A.6.2.21.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1417

A.6.2.21.2.5	Reception of a Correct Message over Temporary C-RNTI	1417

A.6.2.21.2.6	Contention Resolution Timer expiry	1417

A.6.2.21.2.7	NPRACH Resource Selection	1417

A.6.3	RRC Connection Release with Redirection	1418

A.6.3.1	Redirection from E-UTRAN FDD to UTRAN FDD	1418

A.6.3.1.1	Test Purpose and Environment	1418

A.6.3.1.2	Test Requirements	1420

A.6.3.2	Redirection from E-UTRAN TDD to UTRAN FDD	1420

A.6.3.2.1	Test Purpose and Environment	1420

A.6.3.2.2	Test Requirements	1422

A.6.3.3	Redirection from E-UTRAN FDD to GERAN when System Information is provided	1422

A.6.3.3.1	Test Purpose and Environment	1422

A.6.3.3.2	Test Requirements	1423

A.6.3.4	Redirection from E-UTRAN TDD to GERAN when System Information is provided	1424

A.6.3.4.1	Test Purpose and Environment	1424

A.6.3.4.2	Test Requirements	1425

A.6.3.5	E-UTRA TDD RRC connection release redirection to UTRA TDD	1426

A.6.3.5.1	Test Purpose and Environment	1426

A.6.3.5.2	Test Requirements	1428

A.6.3.6	E-UTRA FDD RRC connection release redirection to UTRA TDD	1428

A.6.3.6.1	Test Purpose and Environment	1428

A. 6.3.6.2	Test Requirements	1431

A.6.3.7	E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided	1431

A.6.3.7.1	Test Purpose and Environment	1431

A.6.3.7.2	Test Requirements	1434

A.6.3.8	E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided	1434

A.6.3.8.1	Test Purpose and Environment	1434

A.6.3.8.2	Test Requirements	1437

A.6.3.9	Redirection from E-UTRAN FDD to UTRAN FDD without System Information	1437

A.6.3.9.1	Test Purpose and Environment	1437

A.6.3.9.2	Test Requirements	1439

A.6.3.10	Redirection from E-UTRAN FDD to GERAN when System Information is not provided	1439

A.6.3.10.1	Test Purpose and Environment	1439

A.6.3.10.2	Test Requirements	1441

A.6.3.11	Redirection from E-UTRAN TDD to GERAN when System Information is not provided	1441

A.6.3.11.1	Test Purpose and Environment	1441

A.6.3.11.2	Test Requirements	1443

A.6.3.12	E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided	1443

A.6.3.12.1	Test Purpose and Environment	1443

A.6.3.12.2	Test Requirements	1446

A.7	Timing and Signalling Characteristics	1447

A.7.1	UE Transmit Timing	1447

A.7.1.1	E-UTRAN FDD – UE Transmit Timing Accuracy Tests	1447

A.7.1.1.1	Test Purpose and Environment	1447

A.7.1.1.2	Test Requirements	1448

A.7.1.2	E-UTRAN TDD - UE Transmit Timing Accuracy Tests	1449

A.7.1.2.1	Test Purpose and Environment	1449

A.7.1.2.2	Test Requirements	1451

A.7.1.3	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell	1452

A.7.1.3.1	Test Purpose and Environment	1452

A.7.1.3.2	Test Requirements	1454

A.7.1.4	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell	1455

A.7.1.4.1	Test Purpose and Environment	1455

A.7.1.4.2	Test Requirements	1456

A.7.1.4A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell for 20 MHz + 10 MHz	1457

A.7.1.4A.1	Test Purpose and Environment	1457

A.7.1.4A.2	Test Requirements	1457

A.7.1.5	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for 5MHz Bandwidth	1457

A.7.1.5.1	Test Purpose and Environment	1457

A.7.1.5.2	Test Requirements	1458

A.7.1.6	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell in sTAG	1458

A.7.1.6.1	Test Purpose and Environment	1458

A.7.1.6.2	Test Requirements	1460

A.7.1.7	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG	1461

A.7.1.7.1	Test Purpose and Environment	1461

A.7.1.7.2	Test Requirements	1462

A.7.1.7A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +20MHz	1463

A.7.1.7A.1	Test Purpose and Environment	1463

A.7.1.7A.2	Test Requirements	1463

A.7.1.7B	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +10MHz	1463

A.7.1.7B.1	Test Purpose and Environment	1463

A.7.1.7B.2	Test Requirements	1463

A.7.1.8	Void	1464

A.7.1.8.1	Void	1464

A.7.1.8.2	Void	1464

A.7.1.9	Void	1464

A.7.1.9.1	Void	1464

A.7.1.9.2	Void	1464

A.7.1.10	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1464

A.7.1.10.1	Test Purpose and Environment	1464

A.7.1.10.2	Test Requirements	1466

A.7.1.11	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1466

A.7.1.11.1	Test Purpose and Environment	1466

A.7.1.11.2	Test Requirements	1468

A.7.1.12	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1468

A.7.1.12.1	Test Purpose and Environment	1468

A.7.1.12.2	Test Requirements	1470

A.7.1.13	3DL/3UL TDD CA UE Transmit Timing Accuracy Tests for 2 SCells	1470

A.7.1.13.1	Test Purpose and Environment	1470

A.7.1.13.2	Test Requirements	1472

A.7.1.14	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1473

A.7.1.14.1	Test Purpose and Environment	1473

A.7.1.14.2	Test Requirements	1474

A.7.1.15	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1474

A.7.1.15.1	Test Purpose and Environment	1474

A.7.1.15.2	Test Requirements	1475

A.7.1.16	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1476

A.7.1.16.1	Test Purpose and Environment	1476

A.7.1.16.2	Test Requirements	1477

A.7.1.17	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1478

A.7.1.17.1	Test Purpose and Environment	1478

A.7.1.17.2	Test Requirements	1480

A.7.1.18	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1481

A.7.1.18.1	Test Purpose and Environment	1481

A.7.1.18.2	Test Requirements	1483

A.7.1.19	E-UTRAN FDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1484

A.7.1.19.1	Test Purpose and Environment	1484

A.7.1.19.2	Test Requirements	1485

A.7.1.20	E-UTRAN TDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1485

A.7.1.20.1	Test Purpose and Environment	1485

A.7.1.20.2	Test Requirements	1487

A.7.1.21	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1487

A.7.1.21.1	Test Purpose and Environment	1487

A.7.1.21.2	Test Requirements	1489

A.7.1.22	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1489

A.7.1.22.1	Test Purpose and Environment	1489

A.7.1.22.2	Test Requirements	1491

A.7.1.23	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1491

A.7.1.23.1	Test Purpose and Environment	1491

A.7.1.23.2	Test Requirements	1493

A.7.1.24	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1493

A.7.1.24.1	Test Purpose and Environment	1493

A.7.1.24.2	Test Requirements	1494

A.7.1.25	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1495

A.7.1.25.1	Test Purpose and Environment	1495

A.7.1.25.2	Test Requirements	1496

A.7.1.26	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1497

A.7.1.26.1	Test Purpose and Environment	1497

A.7.1.26.2	Test Requirements	1498

A.7.1.27	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1499

A.7.1.27.1	Test Purpose and Environment	1499

A.7.1.27.2	Test Requirements	1501

A.7.1.28	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1501

A.7.1.28.1	Test Purpose and Environment	1501

A.7.1.28.2	Test Requirements	1503

A.7.2	UE Timing Advance	1504

A.7.2.1	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test	1504

A.7.2.1.1	Test Purpose and Environment	1504

A.7.2.1.2	Test Requirements	1505

A.7.2.2	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test	1506

A.7.2.2.1	Test Purpose and Environment	1506

A.7.2.2.2	Test Requirements	1508

A.7.2.3	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for 5MHz	1508

A.7.2.3.1	Test Purpose and Environment	1508

A.7.2.3.2	Test Requirements	1508

A.7.2.4	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for SCell in sTAG	1508

A.7.2.4.1	Test Purpose and Environment	1508

A.7.2.4.2	Test Requirements	1511

A.7.2.5	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG	1511

A.7.2.5.1	Test Purpose and Environment	1511

A.7.2.5.2	Test Requirements	1513

A.7.2.5A	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +20 MHz	1513

A.7.2.5A.1	Test Purpose and Environment	1513

A.7.2.5A.2	Test Requirements	1514

A.7.2.5B	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +10 MHz	1514

A.7.2.5B.1	Test Purpose and Environment	1514

A.7.2.5B.2	Test Requirements	1514

A.7.2.6	E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1514

A.7.2.6.1	Test Purpose and Environment	1514

A.7.2.6.2	Test Requirements	1517

A.7.2.7	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1517

A.7.2.7.1	Test Purpose and Environment	1517

A.7.2.7.2	Test Requirements	1519

A.7.2.8	E-UTRAN TDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1519

A.7.2.8.1	Test Purpose and Environment	1519

A.7.2.8.2	Test Requirements	1521

A.7.2.9.2	Test Requirements	1522

A.7.2.10	E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1523

A.7.2.10.1	Test Purpose and Environment	1523

A.7.2.10.2	Test Requirements	1524

A.7.2.11	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1524

A.7.2.11.1	Test Purpose and Environment	1524

A.7.2.11.2	Test Requirements	1526

A.7.2.12	E-UTRAN TDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1526

A.7.2.12.1	Test Purpose and Environment	1526

A.7.2.12.2	Test Requirements	1528

A.7.2.13	E-UTRAN FDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1529

A.7.2.13.1	Test Purpose and Environment	1529

A.7.2.13.2	Test Requirements	1530

A.7.2.14	E-UTRAN TDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1531

A.7.2.14.1	Test Purpose and Environment	1531

A.7.2.14.2	Test Requirements	1533

A.7.2.15	E-UTRAN TDD – TDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhanced Coverage	1533

A.7.2.15.1	Test Purpose and Environment	1533

A.7.2.15.2	Test Requirements	1535

A.7.3	Radio Link Monitoring	1535

A.7.3.1	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync	1535

A.7.3.1.1	Test Purpose and Environment	1535

A.7.3.1.2	Test Requirements	1539

A.7.3.2	E-UTRAN FDD Radio Link Monitoring Test for In-sync	1539

A.7.3.2.1	Test Purpose and Environment	1539

A.7.3.2.2	Test Requirements	1543

A.7.3.3	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync	1543

A.7.3.3.1	Test Purpose and Environment	1543

A.7.3.3.2	Test Requirements	1547

A.7.3.4	E-UTRAN TDD Radio Link Monitoring Test for In-sync	1547

A.7.3.4.1	Test Purpose and Environment	1547

A.7.3.4.2	Test Requirements	1551

A.7.3.5	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX	1551

A.7.3.5.1	Test Purpose and Environment	1551

A.7.3.5.2	Test Requirements	1554

A.7.3.6	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX	1555

A.7.3.6.1	Test Purpose and Environment	1555

A.7.3.6.2	Test Requirements	1558

A.7.3.7	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX	1558

A.7.3.7.1	Test Purpose and Environment	1558

A.7.3.7.2	Test Requirements	1561

A.7.3.8	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX	1561

A.7.3.8.1	Test Purpose and Environment	1561

A.7.3.8.2	Test Requirements	1564

A.7.3.9 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction and Non-MBSFN ABS	1564

A.7.3.9.2	Test Requirements	1567

A.7.3.10	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1567

A.7.3.10.1	Test Purpose and Environment	1567

A.7.3.11	E-UTRAN FDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1571

A.7.3.11.1	Test Purpose and Environment	1571

A.7.3.11.2	Test Requirements	1576

A.7.3.12	E-UTRAN TDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1576

A.7.3.12.1	Test Purpose and Environment	1576

A.7.3.12.2	Test Requirements	1581

A.7.3.13 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1581

A.7.3.13.1	Test Purpose and Environment	1581

A.7.3.13.2	Test Requirements	1584

A.7.3.14 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1584

A.7.3.14.1	Test Purpose and Environment	1584

A.7.3.14.2	Test Requirements	1587

A.7.3.15	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1587

A.7.3.15.1	Test Purpose and Environment	1587

A.7.3.15.2	Test Requirements	1591

A.7.3.16	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1591

A.7.3.16.1	Test Purpose and Environment	1591

A.7.3.16.2	Test Requirements	1596

A.7.3.17	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1596

A.7.3.17.1	Test Purpose and Environment	1596

A.7.3.17.2	Test Requirements	1600

A.7.3.18	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1600

A.7.3.18.1	Test Purpose and Environment	1600

A.7.3.18.2	Test Requirements	1604

A.7.3.19	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1604

A.7.3.19.1	Test Purpose and Environment	1604

A.7.3.19.2	Test Requirements	1609

A.7.3.20	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1609

A.7.3.20.1	Test Purpose and Environment	1609

A.7.3.20.2	Test Requirements	1614

A.7.3.21	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1614

A.7.3.21.1	Test Purpose and Environment	1614

A.7.3.21.2	Test Requirements	1619

A.7.3.22	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1619

A.7.3.22.1	Test Purpose and Environment	1619

A.7.3.22.2	Test Requirements	1624

A.7.3.23	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth	1624

A.7.3.23.1	Test Purpose and Environment	1624

A.7.3.23.2	Test Requirements	1625

A.7.3.24	E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth	1625

A.7.3.24.1	Test Purpose and Environment	1625

A.7.3.24.2	Test Requirements	1626

A.7.3.25	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth	1626

A.7.3.25.1	Test Purpose and Environment	1626

A.7.3.25.2	Test Requirements	1627

A.7.3.26	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1627

A.7.3.26.1	Test Purpose and Environment	1627

A.7.3.26.2	Test Requirements	1630

A.7.3.27	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1630

A.7.3.27.1	Test Purpose and Environment	1630

A.7.3.27.2	Test Requirements	1633

A.7.3.28	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1633

A.7.3.28.1	Test Purpose and Environment	1633

A.7.3.28.2	Test Requirements	1636

A.7.3.29	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1636

A.7.3.29.1	Test Purpose and Environment	1636

A.7.3.29.2	Test Requirements	1639

A.7.3.30	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1639

A.7.3.30.1	Test Purpose and Environment	1639

A.7.3.30.2	Test Requirements	1642

A.7.3.31	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1642

A.7.3.31.1	Test Purpose and Environment	1642

A.7.3.31.2	Test Requirements	1645

A.7.3.32	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1645

A.7.3.32.1	Test Purpose and Environment	1645

A.7.3.32.2	Test Requirements	1648

A.7.3.33	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1648

A.7.3.33.1	Test Purpose and Environment	1648

A.7.3.33.2	Test Requirements	1651

A.7.3.34	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1651

A.7.3.34.1	Test Purpose and Environment	1651

A.7.3.34.2	Test Requirements	1654

A.7.3.35	E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE category 0	1654

A.7.3.35.1	Test Purpose and Environment	1654

A.7.3.35.2	Test Requirements	1657

A.7.3.36	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1657

A.7.3.36.1	Test Purpose and Environment	1657

A.7.3.36.2	Test Requirements	1660

A.7.3.37	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE category 0	1660

A.7.3.37.1	Test Purpose and Environment	1660

A.7.3.37.2	Test Requirements	1663

A.7.3.38	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1663

A.7.3.38.1	Test Purpose and Environment	1663

A.7.3.38.2	Test Requirements	1666

A.7.3.39	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC	1666

A.7.3.39.1	Test Purpose and Environment	1666

A.7.3.39.2	Test Requirements	1669

A.7.3.40	E-UTRAN TDD-TDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1670

A.7.3.40.1	Test Purpose and Environment	1670

A.7.3.40.2	Test Requirements	1672

A.7.3.41	E-UTRAN FDD-FDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1673

A.7.3.41.1	Test Purpose and Environment	1673

A.7.3.41.2	Test Requirements	1676

A.7.3.42	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for In-sync in DRX in asynchronous DC	1676

A.7.3.42.1	Test Purpose and Environment	1676

A.7.3.42.2	Test Requirements	1679

A.7.3.43	E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1679

A.7.3.43.1	Test Purpose and Environment	1679

A.7.3.43.2	Test Requirements	1682

A.7.3.44	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in FDD	1682

A.7.3.44.1	Test Purpose and Environment	1682

A.7.3.44.2	Test Requirements	1685

A.7.3.45	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in TDD	1686

A.7.3.45.1	Test Purpose and Environment	1686

A.7.3.45.2	Test Requirements	1688

A.7.3.46	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in FDD	1689

A.7.3.46.1	Test Purpose and Environment	1689

A.7.3.46.2	Test Requirements	1692

A.7.3.47	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in TDD	1692

A.7.3.47.1	Test Purpose and Environment	1692

A.7.3.47.2	Test Requirements	1695

A.7.3.48	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1695

A.7.3.48.1	Test Purpose and Environment	1695

A.7.3.48.2	Test Requirements	1698

A.7.3.49	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1698

A.7.3.49.1	Test Purpose and Environment	1698

A.7.3.49.2	Test Requirements	1701

A.7.3.50	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1701

A.7.3.50.1	Test Purpose and Environment	1701

A.7.3.50.2	Test Requirements	1704

A.7.3.51	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1704

A.7.3.51.1	Test Purpose and Environment	1704

A.7.3.51.2	Test Requirements	1707

A.7.3.52	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1707

A.7.3.52.1	Test Purpose and Environment	1707

A.7.3.52.2	Test Requirements	1710

A.7.3.53	E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1710

A.7.3.53.1	Test Purpose and Environment	1710

A.7.3.53.2	Test Requirements	1713

A.7.3.54	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1713

A.7.3.54.1	Test Purpose and Environment	1713

A.7.3.54.2	Test Requirements	1716

A.7.3.55	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1716

A.7.3.55.1	Test Purpose and Environment	1716

A.7.3.55.2	Test Requirements	1719

A.7.3.56	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1719

A.7.3.56.1	Test Purpose and Environment	1719

A.7.3.56.2	Test Requirements	1722

A.7.3.57	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1722

A.7.3.57.1	Test Purpose and Environment	1722

A.7.3.57.2	Test Requirements	1725

A.7.3.58	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1725

A.7.3.58.1	Test Purpose and Environment	1725

A.7.3.58.2	Test Requirements	1728

A.7.3.59	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1728

A.7.3.59.1	Test Purpose and Environment	1728

A.7.3.59.2	Test Requirements	1731

A.7.3.60	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1731

A.7.3.60.1	Test Purpose and Environment	1731

A.7.3.60.2	Test Requirements	1735

A.7.3.61	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1735

A.7.3.61.1	Test Purpose and Environment	1735

A.7.3.61.2	Test Requirements	1739

A.7.3.62	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1739

A.7.3.62.1	Test Purpose and Environment	1739

A.7.3.62.2	Test Requirements	1743

A.7.3.63	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1743

A.7.3.63.1	Test Purpose and Environment	1743

A.7.3.63.2	Test Requirements	1747

A.7.3.64	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1747

A.7.3.64.1	Test Purpose and Environment	1747

A.7.3.64.2	Test Requirements	1750

A.7.3.65	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1751

A.7.3.65.1	Test Purpose and Environment	1751

A.7.3.65.2	Test Requirements	1754

A.7.3.66	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1755

A.7.3.66.1	Test Purpose and Environment	1755

A.7.3.66.2	Test Requirements	1757

A.7.3.67	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1758

A.7.3.67.1	Test Purpose and Environment	1758

A.7.3.67.2	Test Requirements	1761

A.7.3.68	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1762

A.7.3.68.1	Test Purpose and Environment	1762

A.7.3.68.2	Test Requirements	1764

A.7.3.69	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1764

A.7.3.69.1	Test Purpose and Environment	1764

A.7.3.69.2	Test Requirements	1766

A.7.3.70	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1766

A.7.3.70.1	Test Purpose and Environment	1766

A.7.3.70.2	Test Requirements	1768

A.7.3.71	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1768

A.7.3.71.1	Test Purpose and Environment	1768

A.7.3.71.2	Test Requirements	1771

A.7.3.72	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1771

A.7.3.72.1	Test Purpose and Environment	1771

A.7.3.72.2	Test Requirements	1773

A.7.3.73	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1773

A.7.3.73.1	Test Purpose and Environment	1773

A.7.3.73.2	Test Requirements	1775

A.7.3.74	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1775

A.7.3.74.1	Test Purpose and Environment	1775

A.7.3.74.2	Test Requirements	1778

A.7.3.75	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1778

A.7.3.75.1	Test Purpose and Environment	1778

A.7.3.75.2	Test Requirements	1780

A.7.3.76	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1781

A.7.3.76.1	Test Purpose and Environment	1781

A.7.3.76.2	Test Requirements	1784

A.7.3.77	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1784

A.7.3.77.1	Test Purpose and Environment	1784

A.7.3.77.2	Test Requirements	1787

A.7.3.78	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1787

A.7.3.78.1	Test Purpose and Environment	1787

A.7.3.78.2	Test Requirements	1790

A.7.3.79	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1790

A.7.3.79.1	Test Purpose and Environment	1790

A.7.3.79.2	Test Requirements	1793

A.7.3.80	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1793

A.7.3.80.1	Test Purpose and Environment	1793

A.7.3.80.2	Test Requirements	1796

A.7.3.81	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1796

A.7.3.81.1	Test Purpose and Environment	1796

A.7.3.81.2	Test Requirements	1799

A.7.3.82	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1799

A.7.3.82.1	Test Purpose and Environment	1799

A.7.3.82.2	Test Requirements	1802

A.7.3.83	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1802

A.7.3.83.1	Test Purpose and Environment	1802

A.7.3.83.2	Test Requirements	1803

A.7.3.84	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1804

A.7.3.84.1	Test Purpose and Environment	1804

A.7.3.84.2	Test Requirements	1806

A.7.3.85	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1806

A.7.3.85.1	Test Purpose and Environment	1806

A.7.3.85.2	Test Requirements	1807

A.7.3.86	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1808

A.7.3.86.1	Test Purpose and Environment	1808

A.7.3.86.2	Test Requirements	1810

A.7.3.87	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1810

A.7.3.87.1	Test Purpose and Environment	1810

A.7.3.87.2	Test Requirements	1812

A.7.3.88	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1813

A.7.3.88.1	Test Purpose and Environment	1813

A.7.3.88.2	Test Requirements	1817

A.7.3.89	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1817

A.7.3.89.1	Test Purpose and Environment	1817

A.7.3.89.2	Test Requirements	1821

A.7.3.90	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1821

A.7.3.90.1	Test Purpose and Environment	1821

A.7.3.90.2	Test Requirements	1825

A.7.3.91	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1825

A.7.3.91.1	Test Purpose and Environment	1825

A.7.3.91.2	Test Requirements	1829

A.7.3.92	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1829

A.7.3.92.1	Test Purpose and Environment	1829

A.7.3.92.2	Test Requirements	1832

A.7.3.93	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1833

A.7.3.93.1	Test Purpose and Environment	1833

A.7.3.93.2	Test Requirements	1836

A.7.3.94	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1837

A.7.3.94.1	Test Purpose and Environment	1837

A.7.3.94.2	Test Requirements	1839

A.7.3.95	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1840

A.7.3.95.1	Test Purpose and Environment	1840

A.7.3.95.2	Test Requirements	1843

A.7.3.96	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	1843

A.7.3.96.1	Test Purpose and Environment	1843

A.7.3.97	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	1845

A.7.3.97.1	Test Purpose and Environment	1845

A.7.3.97.2	Test Requirements	1848

A.7.3.98	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	1848

A.7.3.98.1	Test Purpose and Environment	1848

A.7.3.98.2	Test Requirements	1851

A.7.3.99	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	1851

A.7.3.99.1	Test Purpose and Environment	1851

A.7.3.99.2	Test Requirements	1854

A.7.3.100	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	1854

A.7.3.100.1	Test Purpose and Environment	1854

A.7.3.100.2	Test Requirements	1856

A.7.3.101	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	1856

A.7.3.101.1	Test Purpose and Environment	1856

A.7.3.101.2	Test Requirements	1858

A.7.4	Interruption for Dual Connectivity	1858

A.7.4.1	E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1858

A.7.4.1.1	Test Purpose and Environment	1858

A.7.4.1.2	Test Requirements	1861

A.7.4.2	E-UTRAN TDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1861

A.7.4.2.1	Test Purpose and Environment	1861

A.7.4.2.2	Test Requirements	1863

A.7.4.3	E-UTRAN FDD-FDD Interruption at transitions between active and non-active during DRX in asynchronous dual connectivity	1863

A.7.4.3.1	Test Purpose and Environment	1863

A.7.4.3.2	Test Requirements	1865

A.7.4.4	E-UTRAN FDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1865

A.7.4.4.1	Test Purpose and Environment	1865

A.7.4.4.2	Test Requirements	1868

A.7.4.5	E-UTRAN TDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1868

A.7.4.5.1	Test Purpose and Environment	1868

A.7.4.5.2	Test Requirements	1870

A.7.4.6	E-UTRAN FDD-TDD DC interruption at SRS carrier based switching	1870

A.7.4.6.1	Test Purpose and Environment	1870

A.7.4.6.2	Test Requirements	1872

A.7.4.7	E-UTRAN TDD-TDD DC interruption at SRS carrier based switching	1872

A.7.4.7.1	Test Purpose and Environment	1872

A.7.4.7.2	Test Requirements	1876

A.7.5	Proximity-based Services	1876

A.7.5.1	E-UTRAN FDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1876

A.7.5.1.1	Test Purpose and Environment	1876

A.7.5.1.2	Test Requirements	1877

A.7.5.2	E-UTRAN TDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1878

A.7.5.2.1	Test Purpose and Environment	1878

A.7.5.1.2	Test Requirements	1878

A.7.5.3	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery	1879

A.7.5.3.1	Test Purpose and Environment	1879

A.7.5.3.2	Test Requirements	1880

A.7.5.4	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	1881

A.7.5.4.1	Test Purpose and Environment	1881

A.7.5.4.2	Test Requirements	1882

A.7.5.5	E-UTRAN FDD - Interruptions due to ProSe Direct Communication	1883

A.7.5.5.1	Test Purpose and Environment	1883

A.7.5.5.2	Test Requirements	1885

A.7.5.6	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery with discovery period less than 320ms	1886

A.7.5.6.1	Test Purpose and Environment	1886

A.7.5.6.2	Test Requirements	1887

A.7.5.7	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery	1888

A.7.5.7.1	Test Purpose and Environment	1888

A.7.5.7.2	Test Requirements	1890

A.7.5.8	E-UTRAN FDD-FDD - Cell reselection and timing accuracy for ProSe Direct Discovery transmission on non-serving frequency	1890

A.7.5.8.1	Test Purpose and Environment	1890

A.7.5.8.2	Test Requirements	1892

A.7.5.9	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery reception on non-serving frequency	1893

A.7.5.9.1	Test Purpose and Environment	1893

A.7.5.9.2	Test Requirements	1895

A.7.5.10	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery transmission on non-serving frequency	1896

A.7.5.10.1	Test Purpose and Environment	1896

A.7.5.10.2	Test Requirements	1898

A.7.5.11	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Communication on non-serving frequency	1899

A.7.5.11.1	Test Purpose and Environment	1899

A.7.5.11.2	Test Requirements	1901

A.7.5.12	E-UTRAN FDD - Selection / Reselection of ProSe relay UE	1901

A.7.5.12.1	Test Purpose and Environment	1901

A.7.5.12.2	Test Requirements	1905

A.7.6	Interruption for carrier aggregation	1906

A.7.6.1	E-UTRAN FDD-TDD CA interruption at SRS carrier based switching	1906

A.7.6.1.1	Test Purpose and Environment	1906

A.7.6.1.2	Test Requirements	1909

A.7.6.2	E-UTRAN TDD-TDD CA interruption at SRS carrier based switching	1909

A.7.6.2.1	Test Purpose and Environment	1909

A.7.6.2.2	Test Requirements	1912

A.8	UE Measurements Procedures	1912

A.8.1	E-UTRAN FDD Intra-frequency Measurements	1912

A.8.1.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1912

A.8.1.1.1	Test Purpose and Environment	1912

A.8.1.1.2	Test Requirements	1914

A.8.1.2	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1914

A.8.1.2.1	Test Purpose and Environment	1914

A.8.1.2.2	Test Requirements	1916

A.8.1.3	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1916

A.8.1.3.1	Test Purpose and Environment	1916

A.8.1.3.2	Test Requirements	1919

A.8.1.4	Void	1919

A.8.1.5	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1919

A.8.1.5.1	Test Purpose and Environment	1919

A.8.1.5.2	Test Requirements	1921

A.8.1.6	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1921

A.8.1.6.1	Test Purpose and Environment	1921

A.8.1.6.2	Test Requirements	1923

A.8.1.7 	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1923

A.8.1.7.1	Test Purpose and Environment	1923

A.8.1.7.2	Test Requirements	1925

A.8.1.8	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1926

A.8.1.8.1	Test Purpose and Environment	1926

A.8.1.8.2	Test Requirements	1928

A.8.1.9	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for 5MHz bandwidth	1929

A.8.1.9.1	Test Purpose and Environment	1929

A.8.1.9.2	Test Requirements	1929

A.8.1.10	E-UTRAN FDD-FDD Intra-Frequency Event Triggered Reporting under Fading Propagation Conditions in Synchronous Cells with DRX for 5 MHz Bandwidth	1929

A.8.1.10.1	Test Purpose and Environment	1929

A.8.1.10.2	Test Requirements	1930

A.8.1.11	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1930

A.8.1.11.1	Test Purpose and Environment	1930

A.8.1.11.2	Test Requirements	1932

A.8.1.12	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1933

A.8.1.12.1	Test Purpose and Environment	1933

A.8.1.12.2	Test Requirements	1934

A.8.1.13	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1935

A.8.1.13.1	Test Purpose and Environment	1935

A.8.1.13.2	Test Requirements	1937

A.8.1.14	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1937

A.8.1.14.1	Test Purpose and Environment	1937

A.8.1.14.2	Test Requirements	1939

A.8.1.15	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1940

A.8.1.15.1	Test Purpose and Environment	1940

A.8.1.15.2	Test Requirements	1941

A.8.1.16	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1942

A.8.1.16.1	Test Purpose and Environment	1942

A.8.1.16.2	Test Requirements	1944

A.8.1.17	Void	1944

A.8.1.18	Void	1944

A.8.1.19	E-UTRAN FDD-FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1944

A.8.1.19.1	Test Purpose and Environment	1944

A.8.1.19.2	Test Requirements	1946

A.8.1.20	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1947

A.8.1.20.1	Test Purpose and Environment	1947

A.8.1.20.2	Test Requirements	1949

A.8.1.21	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1949

A.8.1.21.1	Test Purpose and Environment	1949

A.8.1.21.2	Test Requirements	1951

A.8.1.22	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1952

A.8.1.22.1	Test Purpose and Environment	1952

A.8.1.22.2	Test Requirements	1954

A.8.1.23	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1954

A.8.1.23.1	Test Purpose and Environment	1954

A.8.1.23.2	Test Requirements	1955

A.8.1.24	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1956

A.8.1.24.1	Test Purpose and Environment	1956

A.8.1.24.2	Test Requirements	1958

A.8.1.25	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1958

A.8.1.25.1	Test Purpose and Environment	1958

A.8.1.25.2	Test Requirements	1960

A.8.1.26	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1960

A.8.1.26.1	Test Purpose and Environment	1960

A.8.1.27	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1963

A.8.1.27.1	Test Purpose and Environment	1963

A.8.1.27.2	Test Requirements	1965

A.8.1.28	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1965

A.8.1.28.1	Test Purpose and Environment	1965

A.8.1.28.2	Test Requirements	1967

A.8.1.29	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1967

A.8.1.29.1	Test Purpose and Environment	1967

A.8.1.29.2	Test Requirements	1969

A.8.1.30	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1970

A.8.1.30.1	Test Purpose and Environment	1970

A.8.1.30.2	Test Requirements	1972

A.8.1.31 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1972

A.8.1.31.1	Test Purpose and Environment	1972

A.8.1.31.2	Test Requirements	1974

A.8.1.32 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1975

A.8.1.32.1	Test Purpose and Environment	1975

A.8.1.32.2	Test Requirements	1976

A.8.1.33	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1977

A.8.1.33.1	Test Purpose and Environment	1977

A.8.1.33.2	Test Requirements	1978

A.8.1.34	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1979

A.8.1.34.1	Test Purpose and Environment	1979

A.8.1.34.2	Test Requirements	1980

A.8.1.35	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1981

A.8.1.35.1	Test Purpose and Environment	1981

A.8.1.35.2	Test Requirements	1982

A.8.1.36	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1983

A.8.1.36.1	Test Purpose and Environment	1983

A.8.1.36.2	Test Requirements	1984

A.8.1.37	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1985

A.8.1.37.1	Test Purpose and Environment	1985

A.8.1.37.2	Test Requirements	1987

A.8.1.38	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1987

A.8.1.38.1	Test Purpose and Environment	1987

A.8.1.38.2	Test Requirements	1988

A.8.1.39	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1989

A.8.1.39.1	Test Purpose and Environment	1989

A.8.1.39.2	Test Requirements	1991

A.8.1.40	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	1991

A.8.1.40.1	Test Purpose and Environment	1991

A.8.1.40.2	Test Requirements	1993

A.8.1.41	E-UTRAN FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1993

A.8.1.41.1	Test Purpose and Environment	1993

A.8.1.41.2	Test Requirement	1995

A.8.1.42	E-UTRAN HD-FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1995

A.8.1.42.1	Test Purpose and Environment	1995

A.8.1.42.2	Test Requirement	1996

A.8.1.43	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16	1996

A.8.1.43.1	Test Purpose and Environment	1996

A.8.1.43.2	Test Requirements	1999

A.8.2	E-UTRAN TDD Intra-frequency Measurements	1999

A.8.2.1	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1999

A.8.2.1.1	Test Purpose and Environment	1999

A.8.2.1.2	Test Requirements	2001

A.8.2.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	2002

A.8.2.2.1	Test Purpose and Environment	2002

A.8.2.2.2	Test Requirements	2004

A.8.2.3	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2004

A.8.2.3.1	Test Purpose and Environment	2004

A.8.2.3.2	Test Requirements	2006

A.8.2.4	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2007

A.8.2.4.1	Test Purpose and Environment	2007

A.8.2.4.2	Test Requirements	2009

A.8.2.5	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2009

A.8.2.5.1	Test Purpose and Environment	2009

A.8.2.5.2	Test Requirements	2011

A.8.2.6	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2012

A.8.2.6.1	Test Purpose and Environment	2012

A.8.2.6.2	Test Requirements	2015

A.8.2.7	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2016

A.8.2.7.1	Test Purpose and Environment	2016

A.8.2.7.2	Test Requirements	2017

A.8.2.8	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2018

A.8.2.8.1	Test Purpose and Environment	2018

A.8.2.8.2	Test Requirements	2020

A.8.2.9	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	2020

A.8.2.9.1	Test Purpose and Environment	2020

A.8.2.9.2	Test Requirements	2022

A.8.2.10	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	2023

A.8.2.10.1	Test Purpose and Environment	2023

A.8.2.10.2	Test Requirements	2025

A.8.2.11	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	2025

A.8.2.11.1	Test Purpose and Environment	2025

A.8.2.11.2	Test Requirements	2027

A.8.2.12	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	2027

A.8.2.12.1	Test Purpose and Environment	2027

A.8.2.12.2	Test Requirements	2029

A.8.2.13	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	2030

A.8.2.13.1	Test Purpose and Environment	2030

A.8.2.13.2	Test Requirements	2032

A.8.2.14	E-UTRAN TDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	2032

A.8.2.14.1	Test Purpose and Environment	2032

A.8.2.14.2	Test Requirement	2034

A.8.2.15	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16	2034

A.8.2.15.1	Test Purpose and Environment	2034

A.8.2.15.2	Test Requirements	2036

A.8.3	E-UTRAN FDD - FDD Inter-frequency Measurements	2036

A.8.3.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2036

A.8.3.1.1	Test Purpose and Environment	2036

A.8.3.1.2	Test Requirements	2038

A.8.3.2	E-UTRAN FDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2038

A.8.3.2.1	Test Purpose and Environment	2038

A.8.3.2.2	Test Requirements	2041

A.8.3.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used	2041

A.8.3.3.1	Test Purpose and Environment	2041

A.8.3.3.2	Test Requirements	2044

A.8.3.4	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2044

A.8.3.4.1	Test Purpose and Environment	2044

A.8.3.4.2	Test Requirements	2046

A.8.3.5	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2046

A.8.3.5.2	Test Requirements	2048

A.8.3.6	E-UTRAN FDD-FDD Inter-frequency event triggered reporting without measurement gaps under AWGN propagation conditions in asynchronous cells	2048

A.8.3.6.1	Test Purpose and Environment	2048

A.8.3.6.2	Test Requirements	2050

A.8.3.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group	2051

A.8.3.7.1	Test Purpose and Environment	2051

A.8.3.7.2	Test Requirements	2053

A.8.3.8	FDD-FDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	2054

A.8.3.8.1	Test Purpose and Environment	2054

A.8.3.8.2	Test Requirements	2057

A.8.3.9	FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	2057

A.8.3.9.1	Test Purpose and Environment	2057

A.8.3.9.2	Test Requirements	2061

A.8.3.10	E-UTRAN FDD-FDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	2062

A.8.3.10.1	Test Purpose and Environment	2062

A.8.3.10.2	Test Requirements	2063

A.8.3.11	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells with burst gap	2063

A.8.3.11.1	Test Purpose and Environment	2063

A.8.3.11.2	Test Requirement	2065

A.8.3.12	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2066

A.8.3.12.1	Test Purpose and Environment	2066

A.8.3.12.2	Test Requirement	2067

A.8.3.13	E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2068

A.8.3.13.1	Test Purpose and Environment	2068

A.8.3.13.2	Test Requirement	2069

A.8.3.14	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2070

A.8.3.14.1	Test Purpose and Environment	2070

A.8.3.14.2	Test Requirement	2071

A.8.3.15	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2072

A.8.3.15.1	Test Purpose and Environment	2072

A.8.3.15.2	Test Requirement	2073

A.8.3.16	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used	2074

A.8.3.16.1	Test Purpose and Environment	2074

A.8.3.16.2	Test Requirement	2076

A.8.3.17	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	2076

A.8.3.17.1	Test Purpose and Environment	2076

A.8.3.17.2	Test Requirement	2079

A.8.3.18	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2079

A.8.3.18.1	Test Purpose and Environment	2079

A.8.3.18.2	Test Requirement	2082

A.8.3.19	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2082

A.8.3.19.1	Test Purpose and Environment	2082

A.8.3.19.2	Test Requirement	2085

A.8.4	E-UTRAN TDD - TDD Inter-frequency Measurements	2085

A.8.4.1	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2085

A.8.4.1.1	Test Purpose and Environment	2085

A.8.4.1.2	Test Requirements	2087

A.8.4.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells	2087

A.8.4.2.1	Test Purpose and Environment	2087

A.8.4.2.2	Test Requirements	2090

A.8.4.3	E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used	2090

A.8.4.3.1	Test Purpose and Environment	2090

A.8.4.3.2	Test Requirements	2092

A.8.4.4	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2093

A.8.4.4.1	Test Purpose and Environment	2093

A.8.4.4.2	Test Requirements	2095

A.8.4.5	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2096

A.8.4.5.1	Test Purpose and Environment	2096

A.8.4.5.2	Test Requirements	2098

A.8.4.6	E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0	2098

A.8.4.6.1	Test Purpose and Environment	2098

A.8.4.6.2	Test Requirements	2099

A.8.4.7	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for Increased Carrier Monitoring without Reduced Performance Group	2099

A.8.4.7.1	Test Purpose and Environment	2099

A.8.4.7.2	Test Requirements	2103

A.8.4.8	TDD-TDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	2103

A.8.4.8.1	Test Purpose and Environment	2103

A.8.4.8.2	Test Requirements	2106

A.8.4.9	TDD-TDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	2106

A.8.4.9.1	Test Purpose and Environment	2106

A.8.4.9.2	Test Requirements	2111

A.8.4.10	E-UTRAN TDD-TDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	2111

A.8.4.10.1	Test Purpose and Environment	2111

A.8.4.10.2	Test Requirements	2113

A.8.4.11	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells with burst gap	2113

A.8.4.11.1	Test Purpose and Environment	2113

A.8.4.11.2	Test Requirement	2115

A.8.4.12	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2115

A.8.4.12.1	Test Purpose and Environment	2115

A.8.4.12.2	Test Requirement	2117

A.8.4.13	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2118

A.8.4.13.1	Test Purpose and Environment	2118

A.8.4.13.2	Test Requirement	2119

A.8.4.14	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	2120

A.8.4.14.1	Test Purpose and Environment	2120

A.8.4.14.2	Test Requirement	2122

A.8.4.15	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2122

A.8.4.15.1	Test Purpose and Environment	2122

A.8.4.15.2	Test Requirement	2125

A.8.5	E-UTRAN FDD - UTRAN FDD Measurements	2125

A.8.5.1	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	2125

A.8.5.1.1	Test Purpose and Environment	2125

A.8.5.1.2	Test Requirements	2127

A.8.5.2	E-UTRAN FDD - UTRAN FDD SON ANR cell search reporting under AWGN propagation conditions	2127

A.8.5.2.1	Test Purpose and Environment	2127

A.8.5.2.2	Test Requirements	2129

A.8.5.3	E-UTRAN FDD-UTRAN FDD event triggered reporting when DRX is used under fading propagation conditions	2129

A.8.5.3.1	Test Purpose and Environment	2129

A.8.5.3.2	Test Requirements	2132

A.8.5.4	E-UTRAN FDD - UTRAN FDD enhanced cell identification under AWGN propagation conditions	2132

A.8.5.4.1	Test Purpose and Environment	2132

A.8.5.4.2	Test Requirements	2134

A.8.5.5	E- UTRAN FDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	2134

A.8.5.5.1	Test Purpose and Environment	2134

A.8.5.5.2	Test Requirements	2137

A.8.5.6	E-UTRAN FDD - UTRAN FDD event triggered reporting without measurement gaps under AWGN propagation conditions	2137

A.8.5.6.1	Test Purpose and Environment	2137

A.8.5.6.2	Test Requirements	2138

A.8.5.7	E-UTRAN FDD - UTRAN FDD Event Triggered Reporting under Fading Propagation Conditions for 5 MHz Bandwidth	2139

A.8.5.7.1	Test Purpose and Environment	2139

A.8.5.7.2	Test Requirements	2139

A.8.5.8	E-UTRA FDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	2139

A.8.5.8.1	Test Purpose and Environment	2139

A.8.5.8.2	Test Requirements	2142

A.8.6	E-UTRAN TDD - UTRAN FDD Measurements	2142

A.8.6.1	E-UTRAN TDD - UTRAN FDD event triggered reporting under fading propagation conditions	2142

A.8.6.1.1	Test Purpose and Environment	2142

A.8.6.1.2	Test Requirements	2144

A.8.6.2	E- UTRAN TDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	2144

A.8.6.2.1	Test Purpose and Environment	2144

A.8.6.2.2	Test Requirements	2147

A.8.6.3	E-UTRA TDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	2147

A.8.6.3.1	Test Purpose and Environment	2147

A.8.6.3.2	Test Requirements	2150

A.8.7	E-UTRAN TDD – UTRAN TDD Measurements	2150

A.8.7.1	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	2150

A.8.7.1.1	Test Purpose and Environment	2150

A.8.7.1.1.1	Void	2150

A.8.7.1.1.2	1.28 Mcps TDD option	2150

A.8.7.1.1.3	Void	2152

A.8.7.1.2	Test Requirements	2152

A.8.7.1.2.1	Void	2152

A.8.7.1.2.2	1.28 Mcps TDD option	2152

A.8.7.1.2.3	Void	2152

A.8.7.2	E-UTRAN TDD-UTRAN TDD cell search when DRX is used under fading propagation conditions	2152

A.8.7.2.1	Test Purpose and Environment	2152

A.8.7.2.2	Test Requirements	2155

A.8.7.3	E-UTRAN TDD - UTRAN TDD SON ANR cell search reporting in AWGN propagation conditions	2156

A.8.7.3.1	Test Purpose and Environment	2156

A.8.7.3.2	Test Parameters	2156

A.8.7.3.3	Test Requirements	2157

A.8.7.4	E-UTRAN TDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	2158

A.8.7.4.1	Test Purpose and Environment	2158

A.8.7.4.2	Test Requirements	2160

A.8.7.5	E-UTRA TDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	2160

A.8.7.5.1	Test Purpose and Environment	2160

A.8.7.5.2	Test Requirements	2162

A.8.7A	E-UTRAN FDD – UTRAN TDD Measurements	2162

A.8.7A.1	E-UTRA FDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	2162

A.8.7A.1.1	Test Purpose and Environment	2162

A.8.7A.1.2	Test Requirements	2165

A.8.8	E-UTRAN FDD – GSM Measurements	2165

A.8.8.1	E-UTRAN FDD – GSM event triggered reporting in AWGN	2165

A.8.8.1.1	Test Purpose and Environment	2165

A.8.8.1.2	Test Requirements	2167

A.8.8.2	E-UTRAN FDD-GSM event triggered reporting when DRX is used in AWGN	2167

A.8.8.2.1	Test Purpose and Environment	2167

A.8.8.2.2	Test Requirements	2169

A.8.8.3	E-UTRAN FDD – GSM event triggered reporting in AWGN with enhanced BSIC identification	2170

A.8.8.3.1	Test Purpose and Environment	2170

A.8.8.3.2	Test Requirements	2171

A.8.9	E-UTRAN FDD - UTRAN TDD measurements	2172

A.8.9.1	E-UTRAN FDD - UTRAN TDD event triggered reporting in fading propagation conditions	2172

A.8.9.1.1	Test Purpose and Environment	2172

A.8.9.1.2	Test Requirements	2173

A.8.9.2	E-UTRAN FDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	2174

A.8.9.2.1	Test Purpose and Environment	2174

A.8.9.2.2	Test Requirements	2176

A.8.10	E-UTRAN TDD – GSM Measurements	2176

A.8.10.1	E-UTRAN TDD – GSM event triggered reporting in AWGN	2176

A.8.10.1.1	Test Purpose and Environment	2176

A.8.10.1.2	Test Requirements	2177

A.8.10.2	E-UTRAN TDD-GSM event triggered reporting when DRX is used in AWGN	2178

A.8.10.2.1	Test Purpose and Environment	2178

A.8.10.2.2	Test Requirements	2180

A.8.11	Monitoring of Multiple Layers	2180

A.8.11.1	Multiple E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions	2180

A.8.11.1.1	Test Purpose and Environment	2180

A.8.11.1.2	Test Requirements	2182

A.8.11.2	E-UTRAN TDD – E-UTRAN TDD and E-UTRAN TDD Inter-frequency event triggered reporting under fading propagation conditions	2183

A.8.11.2.1	Test Purpose and Environment	2183

A.8.11.2.2	Test Requirements	2184

A.8.11.3	E-UTRAN FDD-FDD Inter-frequency and UTRAN FDD event triggered reporting under fading propagation conditions	2185

A.8.11.3.1	Test Purpose and Environment	2185

A.8.11.3.2	Test Requirements	2187

A.8.11.4	InterRAT E-UTRA TDD to E-UTRA TDD and UTRA TDD cell search test case	2187

A.8.11.4.1	Test Purpose and Environment	2187

A.8.11.4.2	Test Requirements	2190

A.8.11.5	Combined E-UTRAN FDD – E-UTRA FDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	2190

A.8.11.5.1	Test Purpose and Environment	2190

A.8.11.5.2	Test Requirements	2192

A.8.11.6	Combined E-UTRAN TDD – E-UTRA TDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	2193

A.8.11.6.1	Test Purpose and Environment	2193

A.8.11.6.2	Test Requirements	2195

A.8.12	RSTD Intra-frequency Measurements	2196

A.8.12.1	E-UTRAN FDD intra-frequency RSTD measurement reporting delay test case	2196

A.8.12.1.1	Test Purpose and Environment	2196

A.8.12.1.2	Test Requirements	2200

A.8.12.1.2A	Test Requirements for UE Category 1bis	2200

A.8.12.2	E-UTRAN TDD intra-frequency RSTD measurement reporting delay test case	2200

A.8.12.2.1	Test Purpose and Environment	2200

A.8.12.2.2	Test Requirements	2205

A.8.12.2.2A	Test Requirements for UE Category 1bis	2205

A.8.12.3	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A	2205

A.8.12.3.1	Test Purpose and Environment	2205

A.8.12.3.2	Test Requirements	2211

A.8.12.4	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A	2211

A.8.12.4.1	Test Purpose and Environment	2211

A.8.12.4.2	Test Requirements	2216

A.8.12.5	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A	2216

A.8.12.5.1	Test Purpose and Environment	2216

A.8.12.5.2	Test Requirements	2221

A.8.12.6	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B	2221

A.8.12.6.1	Test Purpose and Environment	2221

A.8.12.6.2	Test Requirements	2226

A.8.12.7	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B	2226

A.8.12.7.1	Test Purpose and Environment	2226

A.8.12.7.2	Test Requirements	2231

A.8.12.8	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B	2231

A.8.12.8.1	Test Purpose and Environment	2231

A.8.12.8.2	Test Requirements	2236

A.8.12.9	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2236

A.8.12.9.1	Test Purpose and Environment	2236

A.8.12.9.2	Test Requirements	2241

A.8.12.10	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2241

A.8.12.10.1	Test Purpose and Environment	2241

A.8.12.10.2	Test Requirements	2246

A.8.12.11	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2246

A.8.12.11.1	Test Purpose and Environment	2246

A.8.12.11.2	Test Requirements	2251

A.8.12.12	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2251

A.8.12.12.1	Test Purpose and Environment	2251

A.8.12.12.2	Test Requirements	2256

A.8.12.13	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2256

A.8.12.13.1	Test Purpose and Environment	2256

A.8.12.13.2	Test Requirements	2261

A.8.12.14	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2261

A.8.12.14.1	Test Purpose and Environment	2261

A.8.12.14.2	Test Requirements	2266

A.8.13	RSTD Inter-frequency Measurements	2266

A.8.13.1	E-UTRAN FDD-FDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2266

A.8.13.1.1	Test Purpose and Environment	2266

A.8.13.1.2	Test Requirements	2271

A.8.13.1.2A	Test Requirements for UE Category 1bis	2271

A.8.13.2	E-UTRAN TDD-TDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2271

A.8.13.2.1	Test Purpose and Environment	2271

A.8.13.2.2	Test Requirements	2277

A.8.13.2.2A	Test Requirements for UE Category 1bis	2277

A.8.13.3	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A	2277

A.8.13.3.1	Test Purpose and Environment	2277

A.8.13.3.2	Test Requirements	2284

A.8.13.4	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A	2284

A.8.13.4.1	Test Purpose and Environment	2284

A.8.13.4.2	Test Requirements	2290

A.8.13.5	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A	2290

A.8.13.5.1	Test Purpose and Environment	2290

A.8.13.5.2	Test Requirements	2296

A.8.13.6	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B	2296

A.8.13.6.1	Test Purpose and Environment	2296

A.8.13.6.2	Test Requirements	2302

A.8.13.7	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B	2302

A.8.13.7.1	Test Purpose and Environment	2302

A.8.13.7.2	Test Requirements	2308

A.8.13.8	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B	2308

A.8.13.8.1	Test Purpose and Environment	2308

A.8.13.8.2	Test Requirements	2314

A.8.13.9	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2314

A.8.13.9.1	Test Purpose and Environment	2314

A.8.13.9.2	Test Requirements	2320

A.8.13.10	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2320

A.8.13.10.1	Test Purpose and Environment	2320

A.8.13.10.2	Test Requirements	2326

A.8.13.11	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2326

A.8.13.11.1	Test Purpose and Environment	2326

A.8.13.11.2	Test Requirements	2332

A.8.13.12	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2332

A.8.13.12.1	Test Purpose and Environment	2332

A.8.13.12.2	Test Requirements	2338

A.8.13.13	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2338

A.8.13.13.1	Test Purpose and Environment	2338

A.8.13.13.2	Test Requirements	2344

A.8.13.14	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2344

A.8.13.14.1	Test Purpose and Environment	2344

A.8.13.14.2	Test Requirements	2350

A.8.14	E-UTRAN TDD - FDD Inter-frequency Measurements	2350

A.8.14.1	E-UTRAN TDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2350

A.8.14.1.1	Test Purpose and Environment	2350

A.8.14.1.2	Test Requirements	2352

A.8.14.2	E-UTRAN TDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2352

A.8.14.2.1	Test Purpose and Environment	2352

A.8.14.2.2	Test Requirements	2355

A.8.14.3	E-UTRAN TDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2355

A.8.14.3.1	Test Purpose and Environment	2355

A.8.14.3.2	Test Requirements	2357

A.8.15	E-UTRAN FDD - TDD Inter-frequency Measurements	2358

A.8.15.1	E-UTRAN FDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2358

A.8.15.1.1	Test Purpose and Environment	2358

A.8.15.1.2	Test Requirements	2359

A.8.15.2	E-UTRAN FDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2359

A.8.15.2.1	Test Purpose and Environment	2359

A.8.15.2.2	Test Requirements	2362

A.8.15.3	E-UTRAN FDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2362

A.8.15.3.1	Test Purpose and Environment	2362

A.8.15.3.2	Test Requirements	2364

A.8.16	E-UTRAN Carrier Aggregation Measurements	2365

A.8.16.1	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX	2365

A.8.16.1.1	Test Purpose and Environment	2365

A.8.16.1.2	Test Requirements	2367

A.8.16.2	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX	2367

A.8.16.2.1	Test Purpose and Environment	2367

A.8.16.2.2	Test Requirements	2369

A.8.16.3	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2369

A.8.16.3.1	Test Purpose and Environment	2369

A.8.16.3.2	Test Requirements	2371

A.8.16.3A	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with network controlled PCell interruption in non-DRX	2372

A.8.16.3A.1	Test Purpose and Environment	2372

A.8.16.3A.2	Test Requirements	2373

A.8.16.4	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell  with PCell interruption in non-DRX	2374

A.8.16.4.1	Test Purpose and Environment	2374

A.8.16.4.2	Test Requirements	2375

A.8.16.4A	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2376

A.8.16.4A.1	Test Purpose and Environment	2376

A.8.16.4A.2	Test Requirements	2378

A.8.16.5	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2379

A.8.16.5.1	Test Purpose and Environment	2379

A.8.16.5.2	Test Requirements	2379

A.8.16.6	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2379

A.8.16.6.1	Test Purpose and Environment	2379

A.8.16.6.2	Test Requirements	2380

A.8.16.7	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2380

A.8.16.7.1	 Test Purpose and Environment	2380

A.8.16.7.2	 Test Requirements	2381

A.8.16.8	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2381

A.8.16.8.1	 Test Purpose and Environment	2381

A.8.16.8.2	 Test Requirements	2382

A.8.16.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2382

A.8.16.9.1	Test Purpose and Environment	2382

A.8.16.9.2	Test Requirements	2383

A.8.16.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2383

A.8.16.10.1	Test Purpose and Environment	2383

A.8.16.10.2	Test Requirements	2384

A.8.16.11	E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2384

A.8.16.11.1	 Test Purpose and Environment	2384

A.8.16.11.2	 Test Requirements	2385

A.8.16.12	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2385

A.8.16.12.1	 Test Purpose and Environment	2385

A.8.16.12.2	 Test Requirements	2385

A.8.16.13	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 5MHz +5 MHz bandwidth	2385

A.8.16.13.1	Test Purpose and Environment	2385

A.8.16.13.2	Test Requirements	2386

A.8.16.14	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 5 MHz +5 MHz bandwidth	2386

A.8.16.14.1	Test Purpose and Environment	2386

A.8.16.14.2	Test Requirements	2387

A.8.16.15	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5 +5 MHz bandwidth	2387

A.8.16.15.1	 Test Purpose and Environment	2387

A.8.16.7.2	 Test Requirements	2387

A.8.16.16	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5+5 MHz bandwidth	2387

A.8.16.16.1	 Test Purpose and Environment	2387

A.8.16.16.2	 Test Requirements	2388

A.8.16.17	E-UTRAN FDD activation and deactivation of known SCell in non-DRX	2388

A.8.16.17.1	Test Purpose and Environment	2388

A.8.16.17.2	Test Requirements	2390

A.8.16.17A	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 20MHz	2391

A.8.16.17A.1	Test Purpose and Environment	2391

A.8.16.17A.2	Test Requirements	2391

A.8.16.17B	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2391

A.8.16.17B.1	Test Purpose and Environment	2391

A.8.16.17B.2	Test Requirements	2392

A.8.16.17C	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2392

A.8.16.17C.1	Test Purpose and Environment	2392

A.8.16.17C.2	Test Requirements	2392

A.8.16.18	E-UTRAN TDD activation and deactivation of known SCell in non-DRX	2392

A.8.16.18.1	Test Purpose and Environment	2392

A.8.16.18.2	Test Requirements	2394

A.8.16.18A	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz	2395

A.8.16.18A.1	Test Purpose and Environment	2395

A.8.16.18A.2	Test Requirements	2395

A.8.16.18B	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2395

A.8.16.18B.1	Test Purpose and Environment	2395

A.8.16.18B.2	Test Requirements	2396

A.8.16.18C	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2396

A.8.16.18C.1	Test Purpose and Environment	2396

A.8.16.18C.2	Test Requirements	2396

A.8.16.18D	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz + 10MHz	2396

A.8.16.18D.1	Test Purpose and Environment	2396

A.8.16.18D.2	Test Requirements	2397

A.8.16.19	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX	2397

A.8.16.19.1	Test Purpose and Environment	2397

A.8.16.19.2	Test Requirements	2399

A.8.16.19A	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2399

A.8.16.19A.1	Test Purpose and Environment	2399

A.8.16.19A.2	Test Requirements	2400

A.8.16.19B 	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2400

A.8.16.19B.1	Test Purpose and Environment	2400

A.8.16.19B.2	Test Requirements	2400

A.8.16.19C	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2400

A.8.16.19C.1	Test Purpose and Environment	2400

A.8.16.19C.2	Test Requirements	2401

A.8.16.20	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX	2401

A.8.16.20.1	Test Purpose and Environment	2401

A.8.16.20.2	Test Requirements	2403

A.8.16.20A	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2404

A.8.16.20A.1	Test Purpose and Environment	2404

A.8.16.20A.2	Test Requirements	2404

A.8.16.20B	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2404

A.8.16.20B.1	Test Purpose and Environment	2404

A.8.16.20B.2	Test Requirements	2405

A.8.16.20C	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2405

A.8.16.20C.1	Test Purpose and Environment	2405

A.8.16.20C.2	Test Requirements	2405

A.8.16.20D	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz + 10MHz	2405

A.8.16.20D.1	Test Purpose and Environment	2405

A.8.16.20D.2	Test Requirements	2406

A.8.16.21	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20MHz+10MHz	2406

A.8.16.21.1	Test Purpose and Environment	2406

A.8.16.21.2	Test Requirements	2408

A.8.16.22	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 20MHz+10MHz	2408

A.8.16.22.1	Test Purpose and Environment	2408

A.8.16.22.2	Test Requirements	2410

A.8.16.23	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in FDD	2410

A.8.16.23.1	Test Purpose and Environment	2410

A.8.16.23.2	Test Requirements	2413

A.8.16.24	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in TDD	2413

A.8.16.24.1	Test Purpose and Environment	2413

A.8.16.24.2	Test Requirements	2416

A.8.16.25	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in FDD	2416

A.8.16.25.1	Test Purpose and Environment	2416

A.8.16.25.2	Test Requirements	2419

A.8.16.26	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in TDD	2419

A.8.16.26.1	Test Purpose and Environment	2419

A.8.16.26.2	Test Requirements	2422

A.8.16.27	3 DL PCell in FDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2422

A.8.16.27.1	Test Purpose and Environment	2422

A.8.16.27.2	Test Requirements	2427

A.8.16.28	3 DL PCell in TDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2427

A.8.16.28.1	Test Purpose and Environment	2427

A.8.16.28.2	Test Requirements	2432

A.8.16.29	3 DL FDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2432

A.8.16.29.1	Test Purpose and Environment	2432

A.8.16.29.2	Test Requirements	2437

A.8.16.30	3 DL TDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2437

A.8.16.30.1	Test Purpose and Environment	2437

A.8.16.30.2	Test Requirements	2442

A.8.16.31	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in FDD	2442

A.8.16.31.1	Test Purpose and Environment	2442

A.8.16.31.2	Test Requirements	2447

A.8.16.32	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in TDD	2447

A.8.16.32.1	Test Purpose and Environment	2447

A.8.16.32.2	Test Requirements	2452

A.8.16.33	E-UTRAN FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2452

A.8.16.33.1	Test Purpose and Environment	2452

A.8.16.33.2	Test Requirements	2457

A.8.16.34	E-UTRAN TDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2457

A.8.16.34.1	Test Purpose and Environment	2457

A.8.16.34.2	Test Requirements	2462

A.8.16.35	3 DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX	2462

A.8.16.35.1	Test Purpose and Environment	2462

A.8.16.35.2	Test Requirements	2464

A.8.16.36	3 DL PCell in TDD CA Activation and Deactivation of Known SCell in Non-DRX	2465

A.8.16.36.1	Test Purpose and Environment	2465

A.8.16.36.2	Test Requirements	2467

A.8.16.37	3 DL FDD CA activation and deactivation of known SCell in non-DRX	2468

A.8.16.37.1	Test Purpose and Environment	2468

A.8.16.37.2	Test Requirements	2470

A.8.16.38	3 DL TDD CA activation and deactivation of known SCell in non-DRX	2471

A.8.16.38.1	Test Purpose and Environment	2471

A.8.16.38.2	Test Requirements	2473

A.8.16.39	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in FDD	2474

A.8.16.39.1	Test Purpose and Environment	2474

A.8.16.39.2	Test Requirements	2477

A.8.16.40	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in TDD	2477

A.8.16.40.1	Test Purpose and Environment	2477

A.8.16.40.2	Test Requirements	2480

A.8.16.41	3 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2480

A.8.16.41.1	Test Purpose and Environment	2480

A.8.16.41.2	Test Requirements	2482

A.8.16.42	3 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2483

A.8.16.42.1	Test Purpose and Environment	2483

A.8.16.42.2	Test Requirements	2485

A.8.16.43	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in FDD	2486

A.8.16.43.1	Test Purpose and Environment	2486

A.8.16.43.2	Test Requirements	2489

A.8.16.44	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in FDD	2489

A.8.16.44.1	Test Purpose and Environment	2489

A.8.16.44.2	Test Requirements	2492

A.8.16.45	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in TDD	2492

A.8.16.45.1	Test Purpose and Environment	2492

A.8.16.45.2	Test Requirements	2495

A.8.16.46	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in TDD	2495

A.8.16.46.1	Test Purpose and Environment	2495

A.8.16.46.2	Test Requirements	2497

A.8.16.47	2DL/2UL FDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2498

A.8.16.47.1	Test Purpose and Environment	2498

A.8.16.47.2	Test Requirements	2500

A.8.16.48	2DL/2UL TDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2501

A.8.16.48.1	Test Purpose and Environment	2501

A.8.16.48.2	Test Requirements	2503

A.8.16.49	2DL/2UL TDD-FDD CA (FDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2504

A.8.16.49.1	Test Purpose and Environment	2504

A.8.16.49.2	Test Requirements	2507

A.8.16.50	2DL/2UL TDD-FDD CA (TDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2507

A.8.16.50.1	Test Purpose and Environment	2507

A.8.16.50.2	Test Requirements	2509

A.8.16.51	E-UTRAN 4 DL FDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2510

A.8.16.51.1	Test Purpose and Environment	2510

A.8.16.51.2	Test Requirements	2515

A.8.16.52	E-UTRAN 4 DL TDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2515

A.8.16.52.1	Test Purpose and Environment	2515

A.8.16.52.2	Test Requirements	2520

A.8.16.53	4 DL PCell in FDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2520

A.8.16.53.1	Test Purpose and Environment	2520

A.8.16.53.2	Test Requirements	2524

A.8.16.54	4 DL PCell in TDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2524

A.8.16.54.1	Test Purpose and Environment	2524

A.8.16.54.2	Test Requirements	2528

A.8.16.55	E-UTRAN FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2528

A.8.16.55.1	Test Purpose and Environment	2528

A.8.16.55.2	Test Requirements	2534

A.8.16.56	E-UTRAN TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2534

A.8.16.56.1	Test Purpose and Environment	2534

A.8.16.56.2	Test Requirements	2540

A.8.16.57	E-UTRAN FDD 4DL CA activation and deactivation of know SCell in non-DRX	2540

A.8.16.57.1	Test Purpose and Environment	2540

A.8.16.57.2	Test Requirements	2542

A.8.16.58	E-UTRAN TDD 4DL CA activation and deactivation of know SCell in non-DRX	2543

A.8.16.58.1	Test Purpose and Environment	2543

A.8.16.58.2	Test Requirements	2545

A.8.16.59	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2546

A.8.16.59.1	Test Purpose and Environment	2546

A.8.16.59.2	Test Requirements	2550

A.8.16.60	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2551

A.8.16.60.1	Test Purpose and Environment	2551

A.8.16.60.2	Test Requirements	2554

A.8.16.61	E-UTRAN FDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2555

A.8.16.61.1	Test Purpose and Environment	2555

A.8.16.61.2	Test Requirements	2558

A.8.16.62	E-UTRAN TDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2558

A.8.16.62.1	Test Purpose and Environment	2558

A.8.16.62.2	Test Requirements	2560

A.8.16.63	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2561

A.8.16.63.1	Test Purpose and Environment	2561

A.8.16.63.2	Test Requirements	2564

A.8.16.64	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2565

A.8.16.64.1	Test Purpose and Environment	2565

A.8.16.64.2	Test Requirements	2569

A.8.16.65	5 DL FDD-TDD with PCell in FDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2569

A.8.16.65.1	Test Purpose and Environment	2569

A.8.16.65.2	Test Requirements	2574

A.8.16.66	5 DL FDD-TDD with PCell in TDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2574

A.8.16.66.1	Test Purpose and Environment	2574

A.8.16.66.2	Test Requirements	2579

A.8.16.67	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of Unknown SCell in non-DRX	2579

A.8.16.67.1	Test Purpose and Environment	2579

A.8.16.67.2	Test Requirements	2584

A.8.16.68	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of Unknown SCell in non-DRX	2584

A.8.16.68.1	Test Purpose and Environment	2584

A.8.16.68.2	Test Requirements	2589

A.8.16.69	5 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2589

A.8.16.69.1	Test Purpose and Environment	2589

A.8.16.69.2	Test Requirements	2593

A.8.16.70	5 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2593

A.8.16.70.1	Test Purpose and Environment	2593

A.8.16.70.2	Test Requirements	2597

A.8.16.71	5 DL FDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2597

A.8.16.71.1	Test Purpose and Environment	2597

A.8.16.71.2	Test Requirements	2602

A.8.16.72	5 DL TDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2602

A.8.16.72.1	Test Purpose and Environment	2602

A.8.16.72.2	Test Requirements	2607

A.8.16.73	5 DL FDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2607

A.8.16.73.1	Test Purpose and Environment	2607

A.8.16.73.2	Test Requirements	2613

A.8.16.74	5 DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2613

A.8.16.74.1	Test Purpose and Environment	2613

A.8.16.74.2	Test Requirements	2619

A.8.16.75	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of known SCell in non-DRX	2619

A.8.16.75.1	Test Purpose and Environment	2619

A.8.16.75.2	Test Requirements	2625

A.8.16.76	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of known SCell in non-DRX	2625

A.8.16.76.1	Test Purpose and Environment	2625

A.8.16.76.2	Test Requirements	2631

A.8.16.77	5 DL FDD CA activation and deactivation of know SCell in non-DRX	2631

A.8.16.77.1	Test Purpose and Environment	2631

A.8.16.77.2	Test Requirements	2635

A.8.16.78	5 DL TDD CA activation and deactivation of know SCell in non-DRX	2635

A.8.16.78.1	Test Purpose and Environment	2635

A.8.16.78.2	Test Requirements	2637

A.8.16.79	E-UTRAN PCell in FDD FDD-TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2638

A.8.16.79.1	Test Purpose and Environment	2638

A.8.16.79.2	Test Requirements	2645

A.8.16.80	E-UTRAN PCell in TDD TDD-FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2645

A.8.16.80.1	Test Purpose and Environment	2645

A.8.16.80.2	Test Requirements	2652

A.8.16.81	E-UTRAN PCell in FDD FDD-TDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2652

A.8.16.81.1	Test Purpose and Environment	2652

A.8.16.81.2	Test Requirements	2658

A.8.16.82	E-UTRAN PCell in TDD TDD-FDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2658

A.8.16.82.1	Test Purpose and Environment	2658

A.8.16.82.2	Test Requirements	2664

A.8.16.83	3 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2664

A.8.16.83.1	Test Purpose and Environment	2664

A.8.16.83.2	Test Requirements	2669

A.8.16.84	3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2669

A.8.16.84.1	Test Purpose and Environment	2669

A.8.16.84.2	Test Requirements	2675

A.8.16.85	3 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2675

A.8.16.85.1	Test Purpose and Environment	2675

A.8.16.85.2	Test Requirements	2679

A.8.16.86	3 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2679

A.8.16.86.1	Test Purpose and Environment	2679

A.8.16.86.2	Test Requirements	2682

A.8.16.87	4 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2682

A.8.16.87.1	Test Purpose and Environment	2682

A.8.16.87.2	Test Requirements	2688

A.8.16.88	4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2688

A.8.16.88.1	Test Purpose and Environment	2688

A.8.16.88.2	Test Requirements	2696

A.8.16.89	4 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2696

A.8.16.89.1	Test Purpose and Environment	2696

A.8.16.89.2	Test Requirements	2703

A.8.16.90	4 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2703

A.8.16.90.1	Test Purpose and Environment	2703

A.8.16.90.2	Test Requirements	2710

A.8.16.91	5 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2710

A.8.16.91.1	Test Purpose and Environment	2710

A.8.16.91.2	Test Requirements	2716

A.8.16.92	5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2716

A.8.16.92.1	Test Purpose and Environment	2716

A.8.16.92.2	Test Requirements	2724

A.8.16.93	5 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2724

A.8.16.93.1	Test Purpose and Environment	2724

A.8.16.93.2	Test Requirements	2731

A.8.16.94	5 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2731

A.8.16.94.1	Test Purpose and Environment	2731

A.8.16.94.2	Test Requirements	2738

A.8.16.95	6 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2738

A.8.16.95.1	Test Purpose and Environment	2738

A.8.16.95.2	Test Requirements	2746

A.8.16.96	6 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2746

A.8.16.96.1	Test Purpose and Environment	2746

A.8.16.96.2	Test Requirements	2755

A.8.16.97	6 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2755

A.8.16.97.1	Test Purpose and Environment	2755

A.8.16.97.2	Test Requirements	2762

A.8.16.98	6 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2762

A.8.16.98.1	Test Purpose and Environment	2762

A.8.16.98.2	Test Requirements	2770

A.8.16.99	7 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2770

A.8.16.99.1	Test Purpose and Environment	2770

A.8.16.99.2	Test Requirements	2780

A.8.16.100	7 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2780

A.8.16.100.1	Test Purpose and Environment	2780

A.8.16.100.2	Test Requirements	2790

A.8.16.101	7 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2790

A.8.16.101.1	Test Purpose and Environment	2790

A.8.16.101.2	Test Requirements	2800

A.8.16.102	7 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2800

A.8.16.102.1	Test Purpose and Environment	2800

A.8.16.102.2	Test Requirements	2811

A.8.16.103	Hibernation and Activation of Known SCell in Non-DRX with generic duplex modes	2811

A.8.16.103.1	Test Purpose and Environment	2811

A.8.16.103.2	Test Requirements	2816

A.8.16.104	Hibernation and Activation of Unknown SCell in Non-DRX with generic duplex modes	2817

A.8.16.104.1	Test Purpose and Environment	2817

A.8.16.104.2	Test Requirements	2821

A.8.16.105	Idle Mode measurements of inter-frequency CA candidate cells for early reporting	2822

A.8.16.105.1	Test Purpose and Environment	2822

A.8.16.105.2	Test Requirements	2826

A.8.16.106	Direct Activation of Known SCell in Non-DRX with generic duplex modes	2827

A.8.16.106.1	Test Purpose and Environment	2827

A.8.16.106.2	Test Requirements	2831

A.8.16.107	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX with highSpeedEnhMeasFlag2-r16	2831

A.8.16.107.1	Test Purpose and Environment	2831

A.8.16.107.2	Test Requirements	2833

A.8.16.108	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX with highSpeedEnhMeasFlag2-r16	2834

A.8.16.108.1	Test Purpose and Environment	2834

A.8.16.108.2	Test Requirements	2836

A.8.17	RSTD Measurements for E-UTRAN Carrier Aggregation	2837

A.8.17.1	E-UTRAN FDD RSTD measurement reporting delay test case	2837

A.8.17.1.1	Test Purpose and Environment	2837

A.8.17.1.2	Test Requirements	2843

A.8.17.2	E-UTRAN TDD RSTD measurement reporting delay test case	2843

A.8.17.2.1	Test Purpose and Environment	2843

A.8.17.2.2	Test Requirements	2850

A.8.17.3	E-UTRAN FDD RSTD Measurement Reporting Test Case for 20 MHz	2850

A.8.17.3.1	Test Purpose and Environment	2850

A.8.17.3.2	Test Requirements	2851

A.8.17.4	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20 MHz	2851

A.8.17.4.1	Test Purpose and Environment	2851

A.8.17.4.2	Test Requirements	2852

A.8.17.5	E-UTRAN FDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2852

A.8.17.5.1	Test Purpose and Environment	2852

A.8.17.5.2	Test Requirements	2853

A.8.17.6	E-UTRAN TDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2853

A.8.17.6.1	Test Purpose and Environment	2853

A.8.17.6.2	Test Requirements	2854

A.8.17.7	E-UTRAN FDD RSTD Measurement Reporting Test Case for 5 + 5 MHz Bandwidth	2855

A.8.17.7.1	Test Purpose and Environment	2855

A.8.17.7.2	Test Requirements	2855

A.8.17.8	E-UTRAN TDD RSTD Measurement Reporting Test Case for 5+5 MHz bandwidth	2856

A.8.17.8.1	Test Purpose and Environment	2856

A.8.17.8.2	Test Requirements	2856

A.8.17.9	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20MHz+10MHz	2857

A.8.17.9.1	Test Purpose and Environment	2857

A.8.17.9.2	Test Requirements	2858

A.8.17.10	E-UTRAN 3 DL FDD CA RSTD Measurement Reporting Delay Test Case	2858

A.8.17.10.1	Test Purpose and Environment	2858

A.8.17.10.2	Test Requirements	2865

A.8.17.11	E-UTRAN 3 DL TDD CA RSTD Measurement Reporting Delay Test Case	2866

A.8.17.11.1	Test Purpose and Environment	2866

A.8.17.11.2	Test Requirements	2872

A.8.18	E-UTRAN TDD – HRPD Measurements	2873

A.8.18.1	E-UTRAN TDD-HRPD event triggered reporting under fading propagation conditions	2873

A.8.18.1.1	Test Purpose and Environment	2873

A.8.18.1.2	Test Requirements	2875

A.8.19	E-UTRAN TDD – CDMA2000 1X Measurements	2875

A.8.19.1	E-UTRAN TDD – CDMA2000 1X event triggered reporting under fading propagation conditions	2875

A.8.19.1.1	Test Purpose and Environment	2875

A.8.19.1.2	Test Requirements	2876

A.8.20	Inter-frequency/RAT Measurements in CA mode	2877

A.8.20.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2877

A.8.20.1.1	Test Purpose and Environment	2877

A.8.20.1.2	Test Requirements	2878

A.8.20.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2878

A.8.20.2.1	Test Purpose and Environment	2879

A.8.20.2.2	Test Requirements	2880

A.8.20.2A	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth.	2881

A.8.20.2A.1	Test Purpose and Environment	2881

A.8.20.2A.2	Test Requirements	2881

A.8.20.2B	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +10 MHz bandwidth.	2881

A.8.20.2B.1	Test Purpose and Environment	2881

A.8.20.2B.2	Test Requirements	2884

A.8.20.3	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	2884

A.8.20.3.1	Test Purpose and Environment	2884

A.8.20.3.2	Test Requirements	2886

A.8.20.4	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	2886

A.8.20.4.1	Test Purpose and Environment	2886

A.8.20.4.1.1	1.28 Mcps TDD option	2886

A.8.20.4.2	Test Requirements	2888

A.8.20.4.2.1	1.28 Mcps TDD option	2888

A.8.20.4A	E-UTRAN TDD with 20 MHz +20 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2888

A.8.20.4A.1	Test Purpose and Environment	2888

A.8.20.4A.1.1	1.28 Mcps TDD option	2888

A.8.20.4A.2	Test Requirements	2888

A.8.20.4A.2.1	1.28 Mcps TDD option	2888

A.8.20.4B	E-UTRAN TDD with 20 MHz +10 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2889

A.8.20.4B.1	Test Purpose and Environment	2889

A.8.20.4B.1.1	1.28 Mcps TDD option	2889

A.8.20.4B.2	Test Requirements	2891

A.8.20.4B.2.1	1.28 Mcps TDD option	2891

A.8.21	CSG Proximity Indication Testing Case for E-UTRAN FDD – FDD Inter frequency	2891

A.8.21.1	Test Purpose and Environment	2891

A.8.21.2 Test Requirements	2895

A.8.22	E-UTRAN Discovery Signal Measurements	2895

A.8.22.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2895

A.8.22.1.1	Test Purpose and Environment	2895

A.8.22.1.2	Test Requirements	2898

A.8.22.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2898

A.8.22.2.1	Test Purpose and Environment	2898

A.8.22.2.2	Test Requirements	2901

A.8.22.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2901

A.8.22.3.1	Test Purpose and Environment	2901

A.8.22.3.2	Test Requirements	2904

A.8.22.4	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2904

A.8.22.4.1	Test Purpose and Environment	2904

A.8.22.4.2	Test Requirements	2907

A.8.22.5	E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2907

A.8.22.5.1	Test Purpose and Environment	2907

A.8.22.5.2	Test Requirements	2911

A.8.22.6	E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2911

A.8.22.6.1	Test Purpose and Environment	2911

A.8.22.6.2	Test Requirements	2915

A.8.22.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2915

A.8.22.7.1	Test Purpose and Environment	2915

A.8.22.7.2	Test Requirements	2919

A.8.22.8	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation condition in DRX based on CSI-RS based discovery signal	2919

A.8.22.8.1	Test Purpose and Environment	2919

A.8.22.8.2	Test Requirements	2923

A.8.22.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2923

A.8.22.9.1	Test Purpose and Environment	2923

A.8.22.9.2	Test Requirements	2925

A.8.22.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2926

A.8.22.10.1	Test Purpose and Environment	2926

A.8.22.10.2	Test Requirements	2928

A.8.22.11	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2929

A.8.22.11.1	Test Purpose and Environment	2929

A.8.22.11.2	Test Requirements	2932

A.8.22.12	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2932

A.8.22.12.1	Test Purpose and Environment	2932

A.8.22.12.2	Test Requirements	2936

A.8.23	E-UTRAN Dual Connectivity Measurements	2936

A.8.23.1	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2936

A.8.23.1.1	Test Purpose and Environment	2936

A.8.23.1.2	Test Requirements	2939

A.8.23.2	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in asynchronous DC	2939

A.8.23.2.1	Test Purpose and Environment	2939

A.8.23.2.2	Test Requirements	2942

A.8.23.3	E-UTRAN TDD-TDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2942

A.8.23.3.1	Test Purpose and Environment	2942

A.8.23.3.2	Test Requirements	2945

A.8.23.4	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2945

A.8.23.4.1	Test Purpose and Environment	2945

A.8.23.4.2	Test Requirements	2948

A.8.23.5	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in asynchronous DC	2948

A.8.23.5.1	Test Purpose and Environment	2948

A.8.23.5.2	Test Requirements	2951

A.8.23.6	E-UTRAN TDD-TDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2951

A.8.23.6.1	Test Purpose and Environment	2951

A.8.23.6.2	Test Requirements	2954

A.8.23.7	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Synchronous DC	2954

A.8.23.7.1	Test Purpose and Environment	2954

A.8.23.7.2	Test Requirements	2956

A.8.23.8	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Asynchronous DC	2957

A.8.23.8.1	Test Purpose and Environment	2957

A.8.23.8.2	Test Requirements	2959

A.8.23.9	E-UTRAN TDD Addition and Release Delay of known PSCell in Synchronous DC	2960

A.8.23.9.1	Test Purpose and Environment	2960

A.8.23.9.2	Test Requirements	2963

A.8.23.10	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2963

A.8.23.10.1	Test Purpose and Environment	2963

A.8.23.10.2	Test Requirements	2966

A.8.23.11	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2966

A.8.23.11.1	Test Purpose and Environment	2966

A.8.23.11.2	Test Requirements	2969

A.8.23.12	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2969

A.8.23.12.1	Test Purpose and Environment	2969

A.8.23.12.2	Test Requirements	2972

A.8.23.13	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2972

A.8.23.13.1	Test Purpose and Environment	2972

A.8.23.13.2	Test Requirements	2975

A.8.23.14	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in FDD	2975

A.8.23.14.1	Test Purpose and Environment	2975

A.8.23.14.2	Test Requirements	2978

A.8.23.15	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in TDD	2978

A.8.23.15.1	Test Purpose and Environment	2978

A.8.23.15.2	Test Requirements	2981

A.8.23.16	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with no DRX in asynchronous DC	2981

A.8.23.16.1	Test Purpose and Environment	2981

A.8.23.16.2	Test Requirements	2982

A.8.23.17	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with DRX in asynchronous DC	2983

A.8.23.17.1	Test Purpose and Environment	2983

A.8.23.17.2	Test Requirements	2985

A.8.23.18	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2985

A.8.23.18.1	Test Purpose and Environment	2985

A.8.23.18.2	Test Requirements	2986

A.8.23.19	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	2987

A.8.23.19.1	Test Purpose and Environment	2987

A.8.23.19.2	Test Requirements	2988

A.8.23.20	E-UTRAN TDD - TDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2989

A.8.23.20.1	Test Purpose and Environment	2989

A.8.23.20.2	Test Requirements	2990

A.8.23.21	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2991

A.8.23.21.1	Test Purpose and Environment	2991

A.8.23.21.2	Test Requirements	2992

A.8.23.22	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	2993

A.8.23.22.1	Test Purpose and Environment	2993

A.8.23.22.2	Test Requirements	2994

A.8.23.23	E-UTRAN TDD - TDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2995

A.8.23.23.1	Test Purpose and Environment	2995

A.8.23.23.2	Test Requirements	2996

A.8.23.24	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	2997

A.8.23.24.1	Test Purpose and Environment	2997

A.8.23.24.2	Test Requirements	2999

A.8.23.25	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in asynchronous DC	3000

A.8.23.25.1	Test Purpose and Environment	3000

A.8.23.25.2	Test Requirements	3002

A.8.23.26	E-UTRAN TDD-TDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	3003

A.8.23.26.1	Test Purpose and Environment	3003

A.8.23.26.2	Test Requirements	3005

A.8.23.27	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	3006

A.8.23.27.1	Test Purpose and Environment	3006

A.8.23.27.2	Test Requirements	3009

A.8.23.28	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in asynchronous DC	3009

A.8.23.28.1	Test Purpose and Environment	3009

A.8.23.28.2	Test Requirements	3013

A.8.23.29	E-UTRAN TDD-TDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	3013

A.8.23.29.1	Test Purpose and Environment	3013

A.8.23.29.2	Test Requirements	3017

A.8.24	Proximity-based Services	3017

A.8.24.1	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	3017

A.8.24.1.1	Test Purpose and Environment	3017

A.8.24.1.2	Test Requirements	3018

A.8.24.2	E-UTRAN TDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	3019

A.8.24.2.1	Test Purpose and Environment	3019

A.8.24.2.2	Test Requirements	3020

A.8.24.3	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Communication	3020

A.8.24.3.1	Test Purpose and Environment	3021

A.8.24.3.2	Test Requirements	3022

A.8.25	E-UTRAN-WLAN Measurements	3023

A.8.25.1	E-UTRAN FDD-WLAN Event Triggered Reporting in non-DRX under AWGN	3023

A.8.25.1.1	Test Purpose and Environment	3023

A.8.25.1.2	Test Requirements	3025

A.8.25.2	E-UTRAN TDD-WLAN Event Triggered Reporting in non-DRX under AWGN	3025

A.8.25.2.1	Test Purpose and Environment	3025

A.8.25.2.2	Test Requirements	3027

A.8.26 Frame Structure 3 (FS3)	3027

A.8.26.1	E-UTRAN FDD-FS3 Activation and deactivation of known FS3 SCell with FDD PCell in non-DRX	3027

A.8.26.1.1	Test Purpose and Environment	3027

A.8.26.1.2	Test Requirements	3029

A.8.26.2	E-UTRAN TDD-FS3 Activation and deactivation of known FS3 SCell with TDD PCell in non-DRX	3030

A.8.26.2.1	Test Purpose and Environment	3030

A.8.26.2.2	Test Requirements	3032

A.8.26.3	E-UTRAN FDD-FS3 Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3033

A.8.26.3.1	Test Purpose and Environment	3033

A.8.26.3.2	Test Requirements	3036

A.8.26.3A	E-UTRAN FDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3036

A.8.26.3A.1	Test Purpose and Environment	3036

A.8.26.3A.2	Test Requirements	3039

A.8.26.4	E-UTRAN TDD-FS3 Event triggered reporting on deactivated FS3 SCell and TDD PCell interruption in non-DRX	3039

A.8.26.4.1	Test Purpose and Environment	3039

A.8.26.4.2	Test Requirements	3043

A.8.26.4A	E-UTRAN TDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3043

A.8.26.4A.1	Test Purpose and Environment	3043

A.8.26.4A.2	Test Requirements	3047

A.8.26.5	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	3047

A.8.26.5.1	Test Purpose and Environment	3047

A.8.26.5.2	Test Requirements	3050

A.8.26.5A	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	3050

A.8.26.5A.1	Test Purpose and Environment	3050

A.8.26.5A.2	Test Requirements	3054

A.8.26.6	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	3054

A.8.26.6.1	Test Purpose and Environment	3054

A.8.26.6.2	Test Requirements	3058

A.8.26.6A	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	3058

A.8.26.6A.1	Test Purpose and Environment	3058

A.8.26.6A.2	Test Requirements	3062

A.8.26.7	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	3062

A.8.26.7.1	Test Purpose and Environment	3062

A.8.26.7.2	Test Requirements	3065

A.8.26.8	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	3065

A.8.26.8.1	Test Purpose and Environment	3065

A.8.26.8.2	Test Requirements	3069

A.8.26.9	E-UTRAN FDD-FS3 Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	3069

A.8.26.9.1	Test Purpose and Environment	3069

A.8.26.9.2	Test Requirements	3072

A.8.26.10	E-UTRAN TDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	3072

A.8.26.10.1	Test Purpose and Environment	3072

A.8.26.10.2	Test Requirements	3075

A.9	Measurement Performance Requirements	3076

A.9.1	RSRP	3076

A.9.1.1	FDD Intra frequency case	3076

A.9.1.1.1	Test Purpose and Environment	3076

A.9.1.1.2	Test parameters	3076

A.9.1.1.3	Test Requirements	3079

A.9.1.2	TDD Intra frequency case	3079

A.9.1.2.1	Test Purpose and Environment	3079

A.9.1.2.2	Test parameters	3079

A.9.1.2.3	Test Requirements	3080

A.9.1.3	FDD—FDD Inter frequency case	3081

A.9.1.3.1	Test Purpose and Environment	3081

A.9.1.3.2	Test parameters	3081

A.9.1.3.3	Test Requirements	3084

A.9.1.4	TDD—TDD Inter frequency case	3084

A.9.1.4.1	Test Purpose and Environment	3084

A.9.1.4.2	Test parameters	3084

A.9.1.4.3	Test Requirements	3086

A.9.1.5	FDD—TDD Inter frequency case	3087

A.9.1.5.1	Test Purpose and Environment	3087

A.9.1.5.2	Test parameters	3087

A.9.1.5.3	Test Requirements	3088

A.9.1.6	FDD RSRP for E-UTRAN Carrier Aggregation	3089

A.9.1.6.1	Test Purpose and Environment	3089

A.9.1.6.2	Test parameters	3089

A.9.1.6.3	Test Requirements	3092

A.9.1.7	TDD RSRP for E-UTRAN Carrier Aggregation	3092

A.9.1.7.1	Test Purpose and Environment	3092

A.9.1.7.2	Test parameters	3092

A.9.1.7.3	Test Requirements	3095

A.9.1.8	FDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	3095

A.9.1.8.1	Test Purpose and Environment	3095

A.9.1.8.2	Test parameters	3095

A.9.1.8.3	Test Requirements	3099

A.9.1.9	TDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	3099

A.9.1.9.1	Test Purpose and Environment	3099

A.9.1.9.2	Test parameters	3099

A.9.1.9.3	Test Requirements	3101

A.9.1.10	FDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	3102

A.9.1.10.1	Test Purpose and Environment	3102

A.9.1.10.2	Test parameters	3102

A.9.1.10.3	Test Requirements	3105

A.9.1.11	TDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	3105

A.9.1.11.1	Test Purpose and Environment	3105

A.9.1.11.2	Test parameters	3105

A.9.1.11.3	Test Requirements	3109

A.9.1.12	FDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	3109

A.9.1.12.1	Test Purpose and Environment	3109

A.9.1.12.2	Test parameters	3109

A.9.1.12.3	Test Requirements	3110

A.9.1.13	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	3110

A.9.1.13.1	Test Purpose and Environment	3110

A.9.1.13.2	Test parameters	3110

A.9.1.13.3	Test Requirements	3111

A.9.1.14	FDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3111

A.9.1.14.1	Test Purpose and Environment	3111

A.9.1.14.2	Test parameters	3111

A.9.1.14.3	Test Requirements	3115

A.9.1.15	TDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3115

A.9.1.15.1	Test Purpose and Environment	3115

A.9.1.15.2	Test parameters	3115

A.9.1.15.3	Test Requirements	3117

A.9.1.16	FDD Intra frequency case for 5MHz Bandwidth	3118

A.9.1.16.1	Test Purpose and Environment	3118

A.9.1.16.2	Test parameters	3118

A.9.1.16.3	Test Requirements	3119

A.9.1.17	FDD—FDD Inter frequency case for 5MHz Bandwidth	3119

A.9.1.17.1	Test Purpose and Environment	3119

A.9.1.17.2	Test parameters	3120

A.9.1.17.3	Test Requirements	3121

A.9.1.18	FDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	3121

A.9.1.18.1	Test Purpose and Environment	3121

A.9.1.18.2	Test parameters	3121

A.9.1.18.3	Test Requirements	3122

A.9.1.19	TDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	3123

A.9.1.19.1	Test Purpose and Environment	3123

A.9.1.19.2	Test parameters	3123

A.9.1.19.3	Test Requirements	3123

A.9.1.20	FDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	3123

A.9.1.20.1	Test Purpose and Environment	3123

A.9.1.20.2	Test parameters	3123

A.9.1.20.3	Test Requirements	3125

A.9.1.21	TDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	3125

A.9.1.21.1	Test Purpose and Environment	3125

A.9.1.21.2	Test parameters	3125

A.9.1.21.3	Test Requirements	3125

A.9.1.22	RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	3125

A.9.1.22.1	Test Purpose and Environment	3125

A.9.1.22.2	Test parameters	3126

A.9.1.22.3	Test Requirements	3130

A.9.1.23 RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	3130

A.9.1.23.1	Test Purpose and Environment	3130

A.9.1.23.2	Test parameters	3130

A.9.1.23.3	Test Requirements	3134

A.9.1.24	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz + 10MHz	3134

A.9.1.24.1	Test Purpose and Environment	3134

A.9.1.24.2	Test parameters	3134

A.9.1.24.3	Test Requirements	3135

A.9.1.25	FDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3135

A.9.1.25.1	Test Purpose and Environment	3135

A.9.1.25.2	Test parameters	3135

A.9.1.25.3	Test Requirements	3138

A.9.1.26	TDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3138

A.9.1.26.1	Test Purpose and Environment	3138

A.9.1.26.2	Test parameters	3138

A.9.1.26.3	Test Requirements	3141

A.9.1.27	FDD—FDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3141

A.9.1.27.1	Test Purpose and Environment	3141

A.9.1.27.2	Test parameters	3141

A.9.1.27.3	Test Requirements	3144

A.9.1.28	TDD—TDD inter-frequency absolute and relative  RSRP accuracies in CRS based discovery signal	3144

A.9.1.28.1	Test Purpose and Environment	3144

A.9.1.28.2	Test parameters	3144

A.9.1.28.3	Test Requirements	3147

A.9.1.29	FDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3147

A.9.1.29.1	Test Purpose and Environment	3147

A.9.1.29.2	Test parameters	3147

A.9.1.29.3	Test Requirements	3150

A.9.1.30	TDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3150

A.9.1.30.1	Test Purpose and Environment	3150

A.9.1.30.2	Test parameters	3151

A.9.1.30.3	Test Requirements	3154

A.9.1.31	FDD—FDD inter-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3154

A.9.1.31.1	Test Purpose and Environment	3154

A.9.1.31.2	Test parameters	3154

A.9.1.31.3	Test Requirements	3157

A.9.1.32	TDD—TDD inter-frequency absolute and relative  CSI-RSRP accuracies in CSI-RS based discovery signal	3158

A.9.1.32.1	Test Purpose and Environment	3158

A.9.1.32.2	Test parameters	3158

A.9.1.32.3	Test Requirements	3161

A.9.1.33	FDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	3161

A.9.1.33.1	Test Purpose and Environment	3161

A.9.1.33.2	Test parameters	3161

A.9.1.33.3	Test Requirements	3164

A.9.1.34	TDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	3164

A.9.1.34.1	Test Purpose and Environment	3164

A.9.1.34.2	Test parameters	3164

A.9.1.34.3	Test Requirements	3167

A.9.1.35	FDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	3167

A.9.1.35.1	Test Purpose and Environment	3167

A.9.1.35.2	Test parameters	3167

A.9.1.35.3	Test Requirements	3170

A.9.1.36	TDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	3171

A.9.1.36.1	Test Purpose and Environment	3171

A.9.1.36.2	Test parameters	3171

A.9.1.36.3	Test Requirements	3174

A.9.1.37	3 DL PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	3174

A.9.1.37.1	Test Purpose and Environment	3174

A.9.1.37.2	Test parameters	3174

A.9.1.37.3	Test Requirements	3177

A.9.1.38	3 DL PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	3178

A.9.1.38.1	Test Purpose and Environment	3178

A.9.1.38.2	Test parameters	3178

A.9.1.38.3	Test Requirements	3181

A.9.1.39	3 DL FDD RSRP for E-UTRAN in Carrier Aggregation	3182

A.9.1.39.1	Test Purpose and Environment	3182

A.9.1.39.2	Test parameters	3182

A.9.1.39.3	Test Requirements	3188

A.9.1.40	3 DL TDD RSRP for E-UTRAN in Carrier Aggregation	3188

A.9.1.40.1	Test Purpose and Environment	3188

A.9.1.40.2	Test parameters	3189

A.9.1.40.3	Test Requirements	3195

A.9.1.41	FD-FDD RSRP Intra frequency case for UE category 0	3195

A.9.1.41.1	Test Purpose and Environment	3195

A.9.1.41.2	Test parameters	3195

A.9.1.41.3	Test Requirements	3198

A.9.1.42	HD-FDD RSRP Intra frequency case for UE category 0	3198

A.9.1.42.1	Test Purpose and Environment	3198

A.9.1.42.2	Test parameters	3198

A.9.1.42.3	Test Requirements	3201

A.9.1.43	TDD RSRP Intra frequency case for UE category 0	3201

A.9.1.43.1	Test Purpose and Environment	3201

A.9.1.43.2	Test parameters	3201

A.9.1.43.3	Test Requirements	3202

A.9.1.44	4 DL CA PCell in FDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	3203

A.9.1.44.1	Test Purpose and Environment	3203

A.9.1.44.2	Test parameters	3203

A.9.1.44.3	Test Requirements	3208

A.9.1.45	4 DL CA PCell in TDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	3208

A.9.1.45.1	Test Purpose and Environment	3208

A.9.1.45.2	Test parameters	3208

A.9.1.45.3	Test Requirements	2416

A.9.1.46	4 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2416

A.9.1.46.1	Test Purpose and Environment	2416

A.9.1.46.2	Test parameters	2416

A.9.1.46.3	Test Requirements	2425

A.9.1.47	4 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2426

A.9.1.47.1	Test Purpose and Environment	2426

A.9.1.47.2	Test parameters	2426

A.9.1.47.3	Test Requirements	2435

A.9.1.48	5 DL FDD-TDD with PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	2435

A.9.1.48.1	Test Purpose and Environment	2435

A.9.1.48.2	Test parameters	2436

A.9.1.48.3	Test Requirements	2446

A.9.1.49	5 DL FDD-TDD with PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	2447

A.9.1.49.1	Test Purpose and Environment	2447

A.9.1.49.2	Test parameters	2447

A.9.1.49.3	Test Requirements	2457

A.9.1.50	5 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2458

A.9.1.50.1	Test Purpose and Environment	2458

A.9.1.50.2	Test parameters	2458

A.9.1.50.3	Test Requirements	2465

A.9.1.51	5 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2466

A.9.1.51.1	Test Purpose and Environment	2466

A.9.1.51.2	Test parameters	2466

A.9.1.51.3	Test Requirements	2472

A.9.1.52	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2473

A.9.1.52.1	Test Purpose and Environment	2473

A.9.1.52.2	Test parameters	2473

A.9.1.52.3	Test Requirements	2476

A.9.1.52A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2476

A.9.1.52A.1	Test Purpose and Environment	2476

A.9.1.52A.2	Test parameters	2476

A.9.1.52A.3	Test Requirements	2477

A.9.1.53	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2477

A.9.1.53.1	Test Purpose and Environment	2477

A.9.1.53.2	Test parameters	2478

A.9.1.53.3	Test Requirements	2481

A.9.1.53A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2481

A.9.1.53A.1	Test Purpose and Environment	2481

A.9.1.53A.2	Test parameters	2481

A.9.1.53A.3	Test Requirements	2482

A.9.1.54	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2483

A.9.1.54.1	Test Purpose and Environment	2483

A.9.1.54.2	Test parameters	2483

A.9.1.54.3	Test Requirements	2486

A.9.1.55	FS3 Intra frequency absolute and relative RSRP accuracies with FDD PCell	2486

A.9.1.55.1	Test Purpose and Environment	2486

A.9.1.55.2	Test parameters	2486

A.9.1.55.3	Test Requirements	2490

A.9.1.56	FS3 Intra frequency absolute and relative RSRP accuracies with TDD PCell	2490

A.9.1.56.1	Test Purpose and Environment	2490

A.9.1.56.2	Test parameters	2490

A.9.1.56.3	Test Requirements	2493

A.9.1.57	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2494

A.9.1.57.1	Test Purpose and Environment	2494

A.9.1.57.2	Test parameters	2494

A.9.1.57.3	Test Requirements	2497

A.9.1.57A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2497

A.9.1.57A.1	Test Purpose and Environment	2497

A.9.1.57A.2	Test parameters	2497

A.9.1.57A.3	Test Requirements	2498

A.9.1.58	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2498

A.9.1.58.1	Test Purpose and Environment	2498

A.9.1.58.2	Test parameters	2499

A.9.1.58.3	Test Requirements	2502

A.9.1.58A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2502

A.9.1.58A.1	Test Purpose and Environment	2502

A.9.1.58A.2	Test parameters	2502

A.9.1.58A.3	Test Requirements	2503

A.9.1.59	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2503

A.9.1.59.1	Test Purpose and Environment	2503

A.9.1.59.2	Test parameters	2504

A.9.1.59.3	Test Requirements	2506

A.9.1.60	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with FDD PCell	2506

A.9.1.60.1	Test Purpose and Environment	2506

A.9.1.60.2	Test parameters	2506

A.9.1.60.3	Test Requirements	2509

A.9.1.61	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with TDD PCell	2510

A.9.1.61.1	Test Purpose and Environment	2510

A.9.1.61.2	Test parameters	2510

A.9.1.61.3	Test Requirements	2513

A.9.1.62	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2513

A.9.1.62.1	Test Purpose and Environment	2513

A.9.1.62.2	Test parameters	2513

A.9.1.62.3	Test Requirements	2516

A.9.1.63	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2516

A.9.1.53.1	Test Purpose and Environment	2516

A.9.1.53.2	Test parameters	2516

A.9.1.63.3	Test Requirements	2519

A.9.1.64	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2519

A.9.1.64.1	Test Purpose and Environment	2519

A.9.1.64.2	Test parameters	2519

A.9.1.64.3	Test Requirements	2520

A.9.1.65	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2521

A.9.1.65.1	Test Purpose and Environment	2521

A.9.1.65.2	Test parameters	2521

A.9.1.65.3	Test Requirements	2527

A.9.1.66	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2527

A.9.1.66.1	Test Purpose and Environment	2527

A.9.1.66.2	Test parameters	2527

A.9.1.66.3	Test Requirements	2533

A.9.1.67	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2533

A.9.1.67.1	Test Purpose and Environment	2533

A.9.1.67.2	Test parameters	2533

A.9.1.67.3	Test Requirements	2534

A.9.1.68	3 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2535

A.9.1.68.1	Test Purpose and Environment	2535

A.9.1.68.2	Test parameters	2535

A.9.1.68.3	Test Requirements	2539

A.9.1.69	4 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2539

A.9.1.69.1	Test Purpose and Environment	2539

A.9.1.69.2	Test parameters	2539

A.9.1.69.3	Test Requirements	2543

A.9.1.70	5 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2544

A.9.1.70.1	Test Purpose and Environment	2544

A.9.1.70.2	Test parameters	2544

A.9.1.70.3	Test Requirements	2556

A.9.1.71	6 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2557

A.9.1.71.1	Test Purpose and Environment	2557

A.9.1.71.2	Test parameters	2557

A.9.1.71.3	Test Requirements	2569

A.9.1.72	7 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2570

A.9.1.72.1	Test Purpose and Environment	2570

A.9.1.72.2	Test parameters	2570

A.9.1.72.3	Test Requirements	2582

A.9.1.73	FDD Intra frequency case for CA Idle Mode Measurements	2583

A.9.1.73.1	Test Purpose and Environment	2583

A.9.1.73.2	Test parameters	2583

A.9.1.73.3	Test Requirements	2586

A.9.1.74	FDD—FDD Inter frequency case for CA Idle Mode Measurements for overlapping carrier	2586

A.9.1.74.1	Test Purpose and Environment	2586

A.9.1.74.2	Test parameters	2586

A.9.1.74.3	Test Requirements	2587

A.9.1.75	FDD—FDD Inter frequency case for CA Idle Mode Measurements for non-overlapping carrier	2587

A.9.1.75.1	Test Purpose and Environment	2587

A.9.1.75.2	Test parameters	2588

A.9.1.74.3	Test Requirements	2589

A.9.1.76	FD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	2589

A.9.1.76.1	Test Purpose and Environment	2589

A.9.1.76.2	Test parameters	2589

A.9.1.76.3	Test Requirements	2592

A.9.1.77	HD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	2592

A.9.1.77.1	Test Purpose and Environment	2592

A.9.1.77.2	Test parameters	2592

A.9.1.77.3	Test Requirements	2595

A.9.1.78	TDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	2595

A.9.1.78.1	Test Purpose and Environment	2595

A.9.1.78.2	Test parameters	2595

A.9.1.78.3	Test Requirements	2598

A.9.1.79	FD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	2598

A.9.1.79.1	Test Purpose and Environment	2598

A.9.1.79.2	Test parameters	2598

A.9.1.79.3	Test Requirements	2601

A.9.1.80	HD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	2601

A.9.1.80.1	Test Purpose and Environment	2601

A.9.1.80.2	Test parameters	2601

A.9.1.80.3	Test Requirements	2604

A.9.1.81	TDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	2604

A.9.1.81.1	Test Purpose and Environment	2604

A.9.1.78.2	Test parameters	2604

A.9.1.81.3	Test Requirements	2607

A.9.2	RSRQ	2607

A.9.2.1	FDD Intra frequency case	2607

A.9.2.1.1	Test Purpose and Environment	2607

A.9.2.1.2	Test parameters	2607

A.9.2.1.3	Test Requirements	2610

A.9.2.2	TDD Intra frequency case	2610

A.9.2.2.1	Test Purpose and Environment	2610

A.9.2.2.2	Test parameters	2610

A.9.2.2.3	Test Requirements	2612

A.9.2.3	FDD—FDD Inter frequency case	2612

A.9.2.3.1	Test Purpose and Environment	2612

A.9.2.3.2	Test parameters	2612

A.9.2.3.3	Test Requirements	2615

A.9.2.4	TDD—TDD Inter frequency case	2615

A.9.2.4.1	Test Purpose and Environment	2615

A.9.2.4.2	Test parameters	2615

A.9.2.4.3	Test Requirements	2617

A.9.2.4A	FDD—TDD Inter frequency case	2618

A.9.2.4A.1	Test Purpose and Environment	2618

A.9.2.4A.2	Test parameters	2618

A.9.2.4A.3	Test Requirements	2620

A.9.2.5	FDD RSRQ for E-UTRA Carrier Aggregation	2620

A.9.2.5.1	Test Purpose and Environment	2620

A.9.2.5.2	Test parameters	2621

A.9.2.5.3	Test Requirements	2624

A.9.2.6	TDD RSRQ for E-UTRA Carrier Aggregation	2624

A.9.2.6.1	Test Purpose and Environment	2624

A.9.2.6.2	Test parameters	2625

A.9.2.6.3	Test Requirements	2628

A.9.2.7	FDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2628

A.9.2.7.1	Test Purpose and Environment	2628

A.9.2.7.2	Test parameters	2628

A.9.2.7.3	Test Requirements	2632

A.9.2.8	TDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2632

A.9.2.8.1	Test Purpose and Environment	2632

A.9.2.8.2	Test parameters	2632

A.9.2.8.3	Test Requirements	2635

A.9.2.9	FDD RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS	2635

A.9.2.9.1	Test Purpose and Environment	2635

A.9.2.9.2	Test parameters	2635

A.9.2.9.3	Test Requirements	2639

A.9.2.10	TDD Intra frequency case under time domain measurement resource restriction with MBSFN ABS	2639

A.9.2.10.1	Test Purpose and Environment	2639

A.9.2.10.2	Test parameters	2639

A.9.2.10.3	Test Requirements	2643

A.9.2.11	FDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2643

A.9.2.11.1	Test Purpose and Environment	2643

A.9.2.11.2	Test parameters	2643

A.9.2.11.3	Test Requirements	2644

A.9.2.12	TDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2644

A.9.2.12.1	Test Purpose and Environment	2644

A.9.2.12.2	Test parameters	2645

A.9.2.12.3	Test Requirements	2645

A.9.2.13	Void	2646

A.9.2.13.1	Void	2646

A.9.2.13.2	Void	2646

A.9.2.13.3	Void	2646

A.9.2.14	Void	2646

A.9.2.14.1	Void	2646

A.9.2.14.2	Void	2646

A.9.2.14.3	Void	2646

A.9.2.15	FDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2646

A.9.2.15.1	Test Purpose and Environment	2646

A.9.2.15.2	Test parameters	2646

A.9.2.15.3	Test Requirements	2650

A.9.2.16	TDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2650

A.9.2.16.1	Test Purpose and Environment	2650

A.9.2.16.2	Test parameters	2650

A.9.2.16.3	Test Requirements	2655

A.9.2.17	FDD Intra frequency case for 5 MHz bandwidth	2655

A.9.2.17.1	Test Purpose and Environment	2655

A.9.2.17.2	Test parameters	2655

A.9.2.17.3	Test Requirements	2657

A.9.2.18	FDD—FDD Inter frequency case for 5MHz bandwidth	2657

A.9.2.18.1	Test Purpose and Environment	2657

A.9.2.18.2	Test parameters	2657

A.9.2.18.3	Test Requirements	2660

A.9.2.19	FDD-FDD Inter Frequency WB-RSRQ	2660

A.9.2.19.1	Test Purpose and Environment	2660

A.9.2.19.2	Test parameters	2660

A.9.2.19.3	Test Requirements	2662

A.9.2.20	TDD—TDD Inter Frequency WB-RSRQ	2662

A.9.2.20.1	Test Purpose and Environment	2662

A.9.2.20.2	Test parameters	2662

A.9.2.20.3	Test Requirements	2665

A.9.2.21	FDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2665

A.9.2.21.1	Test Purpose and Environment	2665

A.9.2.21.2	Test parameters	2665

A.9.2.21.3	Test Requirements	2668

A.9.2.22	TDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2668

A.9.2.22.1	Test Purpose and Environment	2669

A.9.2.22.2	Test parameters	2669

A.9.2.22.3	Test Requirements	2669

A.9.2.23	FDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2669

A.9.2.23.1	Test Purpose and Environment	2669

A.9.2.23.2	Test parameters	2669

A.9.2.23.3	Test Requirements	2672

A.9.2.24	TDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2672

A.9.2.24.1	Test Purpose and Environment	2672

A.9.2.24.2	Test parameters	2672

A.9.2.24.3	Test Requirements	2673

A.9.2.25	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	2673

A.9.2.25.1	Test Purpose and Environment	2673

A.9.2.25.2	Test parameters	2673

A.9.2.25.3	Test Requirements	2678

A.9.2.26	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	2678

A.9.2.26.1	Test Purpose and Environment	2678

A.9.2.26.2	Test parameters	2678

A.9.2.26.3	Test Requirements	2682

A.9.2.27	TDD RSRQ for E-UTRAN Carrier Aggregation for 20MHz+10MHz	2682

A.9.2.27.1	Test Purpose and Environment	2682

A.9.2.27.2	Test parameters	2682

A.9.2.27.3	Test Requirements	2683

A.9.2.28	FDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2683

A.9.2.28.1	Test Purpose and Environment	2683

A.9.2.28.2	Test parameters	2683

A.9.2.28.3	Test Requirements	2686

A.9.2.29	TDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2686

A.9.2.29.1	Test Purpose and Environment	2686

A.9.2.29.2	Test parameters	2686

A.9.2.29.3	Test Requirements	2689

A.9.2.30	FDD-FDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2689

A.9.2.30.1	Test Purpose and Environment	2689

A.9.2.30.2	Test parameters	2689

A.9.2.30.3	Test Requirements	2692

A.9.2.31	TDD-TDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2692

A.9.2.31.1	Test Purpose and Environment	2692

A.9.2.31.2	Test parameters	2692

A.9.2.31.3	Test Requirements	2695

A.9.2.32	FDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2695

A.9.2.32.1	Test Purpose and Environment	2695

A.9.2.32.2	Test parameters	2695

A.9.2.32.3	Test Requirements	2698

A.9.2.33	TDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2698

A.9.2.33.1	Test Purpose and Environment	2698

A.9.2.33.2	Test parameters	2698

A.9.2.33.3	Test Requirements	2702

A.9.2.34	FDD—FDD Inter frequency new RSRQ	2702

A.9.2.34.1	Test Purpose and Environment	2702

A.9.2.34.2	Test parameters	2702

A.9.2.34.3	Test Requirements	2705

A.9.2.35	TDD—TDD Inter frequency new RSRQ	2705

A.9.2.35.1	Test Purpose and Environment	2705

A.9.2.35.2	Test parameters	2705

A.9.2.35.3	Test Requirements	2708

A.9.2.36	FDD—FDD Inter frequency RSRQ measured on all OFDM symbols	2708

A.9.2.36.1	Test Purpose and Environment	2708

A.9.2.36.2	Test parameters	2708

A.9.2.36.3	Test Requirements	2709

A.9.2.37	TDD—TDD Inter frequency RSRQ measurement on all OFDM symbols	2710

A.9.2.37.1	Test Purpose and Environment	2710

A.9.2.37.2	Test parameters	2710

A.9.2.37.3	Test Requirements	2711

A.9.2.38	3 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2712

A.9.2.38.1	Test Purpose and Environment	2712

A.9.2.38.2	Test parameters	2712

A.9.2.38.3	Test Requirements	2716

A.9.2.39	3 DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2716

A.9.2.39.1	Test Purpose and Environment	2716

A.9.2.39.2	Test parameters	2716

A.9.2.39.3	Test Requirements	2720

A.9.2.40	3 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2720

A.9.2.40.1	Test Purpose and Environment	2720

A.9.2.40.2	Test parameters	2720

A.9.2.40.3	Test Requirements	2723

A.9.2.41	3 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2724

A.9.2.41.1	Test Purpose and Environment	2724

A.9.2.41.2	Test parameters	2724

A.9.2.41.3	Test Requirements	2727

A.9.2.42	FD-FDD RSRQ Intra frequency case for UE category 0	2727

A.9.2.42.1	Test Purpose and Environment	2727

A.9.2.42.2	Test parameters	2727

A.9.2.42.3	Test Requirements	2730

A.9.2.43	HD-FDD RSRQ Intra frequency case for UE category 0	2730

A.9.2.43.1	Test Purpose and Environment	2730

A.9.2.43.2	Test parameters	2730

A.9.2.43.3	Test Requirements	2733

A.9.2.44	TDD RSRQ Intra frequency case for UE category 0	2733

A.9.2.44.1	Test Purpose and Environment	2733

A.9.2.44.2	Test parameters	2733

A.9.2.44.3	Test Requirements	2735

A.9.2.45	4 DL CA PCell in FDD FDD-TDD RSRQ for E-UTRAN in Carrier Aggregation	2735

A.9.2.45.1	Test Purpose and Environment	2735

A.9.2.45.2	Test parameters	2735

A.9.2.45.3	Test Requirements	2739

A.9.2.46	4 DL CA PCell in TDD TDD-FDD RSRQ for E-UTRAN in Carrier Aggregation	2740

A.9.2.46.1	Test Purpose and Environment	2740

A.9.2.46.2	Test parameters	2740

A.9.2.46.3	Test Requirements	2744

A.9.2.47	5 DL FDD-TDD with PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2745

A.9.2.47.1	Test Purpose and Environment	2745

A.9.2.47.2	Test parameters	2745

A.9.2.47.3	Test Requirements	2749

A.9.2.48	5 DL FDD-TDD with PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2749

A.9.2.48.1	Test Purpose and Environment	2749

A.9.2.48.2	Test parameters	2750

A.9.2.48.3	Test Requirements	2754

A.9.2.49	5 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2755

A.9.2.49.1	Test Purpose and Environment	2755

A.9.2.49.2	Test parameters	2755

A.9.2.49.3	Test Requirements	2761

A.9.2.50	5 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2762

A.9.2.50.1	Test Purpose and Environment	2762

A.9.2.50.2	Test parameters	2762

A.9.2.50.3	Test Requirements	2768

A.9.2.51	FS3 Intra frequency absolute and relative RSRQ accuracies with FDD PCell	2768

A.9.2.51.1	Test Purpose and Environment	2768

A.9.2.51.2	Test parameters	2769

A.9.2.51.3	Test Requirements	2773

A.9.2.52	FS3 Intra frequency absolute and relative RSRQ accuracies with TDD PCell	2773

A.9.2.52.1	Test Purpose and Environment	2773

A.9.2.52.2	Test parameters	2773

A.9.2.52.3	Test Requirements	2776

A.9.2.53	4DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2777

A.9.2.53.1	Test Purpose and Environment	2777

A.9.2.53.2	Test parameters	2777

A.9.2.53.3	Test Requirements	2783

A.9.2.54	4DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2784

A.9.2.54.1	Test Purpose and Environment	2784

A.9.2.54.2	Test parameters	2784

A.9.2.54.3	Test Requirements	2790

A.9.2.55	3 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2790

A.9.2.55.1	Test Purpose and Environment	2790

A.9.2.55.2	Test parameters	2791

A.9.2.55.3	Test Requirements	2796

A.9.2.56	4 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2796

A.9.2.56.1	Test Purpose and Environment	2796

A.9.2.56.2	Test parameters	2796

A.9.2.56.3	Test Requirements	2801

A.9.2.57	5 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2801

A.9.2.57.1	Test Purpose and Environment	2801

A.9.2.57.2	Test parameters	2802

A.9.2.57.3	Test Requirements	2806

A.9.2.58	6 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2807

A.9.2.58.1	Test Purpose and Environment	2807

A.9.2.58.2	Test parameters	2807

A.9.2.58.3	Test Requirements	2811

A.9.2.59	7 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2812

A.9.2.59.1	Test Purpose and Environment	2812

A.9.2.59.2	Test parameters	2812

A.9.2.59.3	Test Requirements	2817

A.9.2.60	FDD Intra frequency case for CA Idle Mode Measurements	2818

A.9.2.60.1	Test Purpose and Environment	2818

A.9.2.60.2	Test parameters	2818

A.9.2.60.3	Test Requirements	2821

A.9.2.61	FDD—FDD Inter frequency case for CA Idle Mode Measurements on overlapping carrier	2821

A.9.2.61.1	Test Purpose and Environment	2821

A.9.2.61.2	Test parameters	2821

A.9.2.61.3	Test Requirements	2824

A.9.2.62	FDD—FDD Inter frequency case for CA Idle Mode Measurements on non-overlapping carrier	2824

A.9.2.62.1	Test Purpose and Environment	2824

A.9.2.62.2	Test parameters	2824

A.9.2.62.3	Test Requirements	2827

A.9.3	UTRAN FDD CPICH RSCP	2827

A.9.3.1	E-UTRAN FDD	2827

A.9.3.1.1	Test Purpose and Environment	2827

A.9.3.1.2	Parameters	2827

A.9.3.1.3	Test Requirements	2830

A.9.3.2 E-UTRAN TDD	2830

A.9.3.2.1	Test Purpose and Environment	2830

A.9.3.2.2	Parameters	2830

A.9.3.2.3	Test Requirements	2833

A.9.3.3	E-UTRAN FDD for 5MHz Bandwidth	2833

A.9.3.3.1	Test Purpose and Environment	2833

A.9.3.3.2	Parameters	2833

A.9.3.3.3	Test Requirements	2834

A.9.4	UTRAN FDD CPICH Ec/No	2835

A.9.4.1	E-UTRAN FDD	2835

A.9.4.1.1	Test Purpose and Environment	2835

A.9.4.1.2	Parameters	2835

A.9.4.1.3	Test Requirements	2837

A.9.4.2	E-UTRAN TDD	2838

A.9.4.2.1	Test Purpose and Environment	2838

A.9.4.2.2	Parameters	2838

A.9.4.2.3	Test Requirements	2840

A.9.4.3	E-UTRAN FDD for 5MHz Bandwidth	2841

A.9.4.3.1	Test Purpose and Environment	2841

A.9.4.3.2	Parameters	2841

A.9.4.3.3	Test Requirements	2842

A.9.5	UTRAN TDD measurement	2842

A.9.5.1	P-CCPCH RSCP absolute accuracy for E-UTRAN FDD	2842

A.9.5.1.1	Test Purpose and Environment	2842

A.9.5.1.2	Test parameters	2842

A.9.5.1.3	Test Requirements	2844

A.9.5.2	P-CCPCH RSCP absolute accuracy for E-UTRAN TDD	2844

A.9.5.2.1	Test Purpose and Environment	2844

A.9.5.2.2	Test parameters	2844

A.9.5.2.3	Test Requirements	2846

A.9.6	GSM Carrier RSSI	2847

A.9.6.1	E-UTRAN FDD	2847

A.9.6.1.1	Test Purpose and Environment	2847

A.9.6.1.2	Test Requirements	2848

A.9.6.2	E-UTRAN TDD	2848

A.9.6.2.1	Test Purpose and Environment	2848

A.9.6.2.2	Test Requirements	2850

A.9.7	UE Rx – Tx Time Difference	2851

A.9.7.1	E-UTRAN FDD UE Rx – Tx time difference case	2851

A.9.7.1.1	Test Purpose and Environment	2851

A.9.7.1.2	Test parameters	2851

A.9.7.1.3	Test Requirements	2852

A.9.7.2	E-UTRA TDD UE Rx – Tx time difference case	2852

A.9.7.2.1	Test Purpose and Environment	2852

A.9.7.2.2	Test parameters	2852

A.9.7.2.3	Test Requirements	2854

A.9.7.3	E-UTRAN FDD UE Rx–Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2854

A.9.7.3.1	Test Purpose and Environment	2854

A.9.7.3.2	Test parameters	2854

A.9.7.3.3	Test Requirements	2857

A.9.7.4	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2857

A.9.7.4.1	Test Purpose and Environment	2857

A.9.7.4.2	Test Parameters	2857

A.9.7.4.3	Test Requirements	2860

A.9.7.5	E-UTRAN FDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2860

A.9.7.5.1	Test Purpose and Environment	2860

A.9.7.5.2	Test parameters	2860

A.9.7.5.3	Test Requirements	2863

A.9.7.6	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2863

A.9.7.6.1	Test Purpose and Environment	2863

A.9.7.6.2	Test Parameters	2863

A.9.7.6.3	Test Requirements	2866

A.9.7.7	E-UTRAN FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2866

A.9.7.7.1	Test Purpose and Environment	2866

A.9.7.7.2	Test parameters	2866

A.9.7.7.3	Test Requirements	2867

A.9.7.8	E-UTRAN HD-FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2868

A.9.7.8.1	Test Purpose and Environment	2868

A.9.7.8.2	Test parameters	2868

A.9.7.8.3	Test Requirements	2869

A.9.7.9	E-UTRAN TDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2869

A.9.7.9.1	Test Purpose and Environment	2869

A.9.7.9.2	Test parameters	2869

A.9.7.9.3	Test Requirements	2870

A.9.8	RSTD	2871

A.9.8.1	E-UTRAN FDD RSTD intra frequency case	2871

A.9.8.1.1	Test Purpose and Environment	2871

A.9.8.1.2	Test Requirements	2873

A.9.8.1.2A	Test Requirements for UE Category 1bis	2873

A.9.8.2	E-UTRAN TDD RSTD intra frequency case	2874

A.9.8.2.1	Test Purpose and Environment	2874

A.9.8.2.2	Test Requirements	2877

A.9.8.2.2A	Test Requirements for UE Category 1bis	2877

A.9.8.3	E-UTRAN FDD-FDD RSTD inter frequency case	2878

A.9.8.3.1	Test Purpose and Environment	2878

A.9.8.3.2	Test Requirements	2880

A.9.8.3.2A	Test Requirements for UE Category 1bis	2880

A.9.8.4	E-UTRAN TDD-TDD RSTD inter frequency case	2881

A.9.8.4.1	Test Purpose and Environment	2881

A.9.8.4.2	Test Requirements	2883

A.9.8.4.2A	Test Requirements for UE Category 1bis	2883

A.9.8.5	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation	2884

A.9.8.5.1	Test Purpose and Environment	2884

A.9.8.5.2	Test Requirements	2886

A.9.8.6	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation	2886

A.9.8.6.1	Test Purpose and Environment	2886

A.9.8.6.2	Test Requirements	2889

A.9.8.7	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2889

A.9.8.7.1	Test Purpose and Environment	2889

A.9.8.7.2	Test Requirements	2890

A.9.8.8	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2890

A.9.8.8.1	Test Purpose and Environment	2890

A.9.8.8.2	Test Requirements	2891

A.9.8.9	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2891

A.9.8.9.1	Test Purpose and Environment	2891

A.9.8.9.2	Test Requirements	2892

A.9.8.10	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2892

A.9.8.10.1	Test Purpose and Environment	2892

A.9.8.10.2	Test Requirements	2893

A.9.8.11	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 5 + 5MHz bandwidth	2893

A.9.8.11.1	Test Purpose and Environment	2893

A.9.8.11.2	Test Requirements	2894

A.9.8.12	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 5+5MHz bandwidth	2894

A.9.8.12.1	Test Purpose and Environment	2894

A.9.8.12.2	Test Requirements	2895

A.9.8.13	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz+10MHz	2895

A.9.8.13.1	Test Purpose and Environment	2895

A.9.8.13.2	Test Requirements	2896

A.9.8.14	E-UTRAN FDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2896

A.9.8.14.1	Test Purpose and Environment	2896

A.9.8.14.2	Test Requirements	2903

A.9.8.15	E-UTRAN TDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2903

A.9.8.15.1	Test Purpose and Environment	2903

A.9.8.15.2	Test Requirements	2909

A.9.8.16	HD – FDD Intra frequency case for UE Category NB1 inband mode in normal coverage	2909

A.9.8.16.1	Test Purpose and Environment	2909

A.9.8.16.2	Test Requirements	2912

A.9.8.17	HD – FDD Inter frequency case for UE Category NB1 inband mode in normal coverage	2912

A.9.8.17.1	Test Purpose and Environment	2912

A.9.8.17.2	Test Requirements	2915

A.9.8.18	HD – FDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	2915

A.9.8.18.1	Test Purpose and Environment	2915

A.9.8.18.2	Test Requirements	2918

A.9.8.19	HD – FDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	2918

A.9.8.19.1	Test Purpose and Environment	2918

A.9.8.19.2	Test Requirements	2921

A.9.8.20	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode A	2921

A.9.8.20.1	Test Purpose and Environment	2921

A.9.8.20.2	Test Requirements	2924

A.9.8.21	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CEModeA	2924

A.9.8.21.1	Test Purpose and Environment	2924

A.9.8.21.2	Test Requirements	2928

A.9.8.22	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode A	2929

A.9.8.22.1	Test Purpose and Environment	2929

A.9.8.22.2	Test Requirements	2932

A.9.8.23	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode B	2933

A.9.8.23.1	Test Purpose and Environment	2933

A.9.8.23.2	Test Requirements	2936

A.9.8.24	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CE Mode B	2936

A.9.8.24.1	Test Purpose and Environment	2936

A.9.8.24.2	Test Requirements	2940

A.9.8.25	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode B	2940

A.9.8.25.1	Test Purpose and Environment	2940

A.9.8.25.2	Test Requirements	2944

A.9.8.26	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	2945

A.9.8.26.1	Test Purpose and Environment	2945

A.9.8.26.2	Test Requirements	2947

A.9.8.27	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	2947

A.9.8.27.1	Test Purpose and Environment	2947

A.9.8.27.2	Test Requirements	2950

A.9.8.28	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode A	2950

A.9.8.28.1	Test Purpose and Environment	2950

A.9.8.28.2	Test Requirements	2953

A.9.8.29	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	2953

A.9.8.29.1	Test Purpose and Environment	2953

A.9.8.29.2	Test Requirements	2957

A.9.8.30	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	2958

A.9.8.30.1	Test Purpose and Environment	2958

A.9.8.30.2	Test Requirements	2961

A.9.8.31	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode B	2962

A.9.8.31.1	Test Purpose and Environment	2962

A.9.8.31.2	Test Requirements	2966

A.9.8.32	TDD Intra frequency case for UE Category NB1 inband mode in normal coverage	2966

A.9.8.32.1	Test Purpose and Environment	2966

A.9.8.32.2	Test Requirements	2969

A.9.8.33	TDD Inter frequency case for UE Category NB1 inband mode in normal coverage	2969

A.9.8.33.1	Test Purpose and Environment	2969

A.9.8.33.2	Test Requirements	2972

A.9.8.34	TDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	2972

A.9.8.34.1	Test Purpose and Environment	2972

A.9.8.34.2	Test Requirements	2975

A.9.8.35	TDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	2975

A.9.8.35.1	Test Purpose and Environment	2975

A.9.8.35.2	Test Requirements	2978

A.9.9	RSRP and RSRQ on the serving cell	2978

A.9.9.1	FDD Intra frequency serving cell case	2978

A.9.9.1.1	Test Purpose and Environment	2978

A.9.9.1.2	Test parameters	2978

A.9.9.1.3	Test Requirements	2981

A.9.9.2	TDD Intra frequency serving cell case	2981

A.9.9.2.1	Test Purpose and Environment	2981

A.9.9.2.2	Test parameters	2981

A.9.9.2.3	Test Requirements	2983

A.9.10	SSTD	2983

A.9.10.1	EUTRAN FDD-FDD SSTD accuracy in asynchronous DC	2983

A.9.10.1.1	Test Purpose and Environment	2983

A.9.10.1.2	Test parameters	2983

A.9.10.1.3	Test Requirements	2985

A.9.10.2	Void	2985

A.9.10.3	Void	2985

A.9.10.4	Void	2985

A.9.11 RSSI	2985

A.9.11.1	FS3 average RSSI accuracy case (PCell using FDD)	2985

A.9.11.1.1	Test Purpose and Environment	2985

A.9.11.1.2	Test parameters	2985

A.9.11.1.3	Test Requirements	2988

A.9.11.2	FS3 average RSSI accuracy case (PCell using TDD)	2988

A.9.11.2.1	Test Purpose and Environment	2988

A.9.11.2.2	Test parameters	2988

A.9.12	Channel occupancy	2991

A.9.12.1	FS3 channel occupancy test (PCell using FDD)	2991

A.9.12.1.1	Test Purpose and Environment	2991

A.9.12.1.2	Test parameters	2991

A.9.12.1.3	Test Requirements	2994

A.9.12.2	FS3 channel occupancy test (PCell using TDD)	2994

A.9.12.2.1	Test Purpose and Environment	2994

A.9.12.2.2	Test parameters	2994

A.9.12.2.3	Test Requirements	2997

A.9.13	RS-SINR	2997

A.9.13.1	FDD Intra-Frequency Case	2997

A.9.13.1.1	Test Purpose and Environment	2997

A.9.13.1.2	Test parameters	2997

A.9.13.1.3	Test Requirements	3000

A.9.13.2	TDD Intra-Frequency Case	3000

A.9.13.2.1	Test Purpose and Environment	3000

A.9.13.2.2	Test parameters	3000

A.9.13.2.3	Test Requirements	3003

A.9.13.3	FDD—FDD Inter frequency case	3003

A.9.13.3.1	Test Purpose and Environment	3003

A.9.13.3.2	Test parameters	3003

A.9.13.3.3	Test Requirements	3006

A.9.13.4	TDD—TDD Inter frequency case	3006

A.9.13.4.1	Test Purpose and Environment	3006

A.9.13.4.2	Test parameters	3006

A.9.13.4.3	Test Requirements	3010

A.9.13.5	FDD—TDD Inter frequency case	3011

A.9.13.5.1	Test Purpose and Environment	3011

A.9.13.5.2	Test parameters	3011

A.9.13.5.3	Test Requirements	3017

A.9.13.6	TDD—FDD Inter frequency case	3017

A.9.13.6.1	Test Purpose and Environment	3017

A.9.13.6.2	Test parameters	3017

A.9.13.6.3	Test Requirements	3021

A.9.14	Channel quality reporting accuracy	3021

A.9.14.1	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage	3021

A.9.14.1.1	Test Purpose and Environment	3021

A.9.14.1.2	Test parameters	3022

A.9.14.1.3	Test Requirements	3022

A.9.14.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage	3022

A.9.14.2.1	Test Purpose and Environment	3022

A.9.14.2.2	Test parameters	3023

A.9.14.2.3	Test Requirements	3023

A.9.14.3	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under normal coverage	3024

A.9.14.3.1	Test Purpose and Environment	3024

A.9.14.3.2	Test parameters	3024

A.9.14.3.3	Test Requirements	3025

A.9.14.4	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under enhanced coverage	3025

A.9.14.4.1	Test Purpose and Environment	3025

A.9.14.4.2	Test parameters	3025

A.9.14.4.3	Test Requirements	3026

A.10	Proximity-based Services in Any Cell Selection State	3049

A.10.1	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	3049

A.10.1.1	Test Purpose and Environment	3049

A.10.1.2	Test Requirements	3050

A.10.2	E-UTRAN FDD – Initiation/Cease of SLSS Transmission with ProSe Direct Communication	3051

A.10.2.1	Test Purpose and Environment	3051

A.10.2.2	Test Requirements	3052

A.10.3	E-UTRAN FDD – SyncRef UE Selection / Reselection Test	3052

A.10.3.1	Test Purpose and Environment	3052

A.10.3.2	Test Requirements	3054

A.10.4	E-UTRAN FDD – Cell Identification on downlink frequency associated with ProSe frequency (when UE is transmitting for ProSe)	3055

A.10.4.1	Test Purpose and Environment	3055

A.10.4.2	Test Requirements	3057

A.11	V2V Sidelink Communication for V2V Operation on Dedicated V2V Carrier	3057

A.11.1	V2V UE Transmission Timing Accuracy Test	3057

A.11.1.1	Test Purpose and Environment	3057

A.11.1.2	Test requirements	3058

A.11.2	Interruptions due to V2V sidelink communication	3058

A.11.2.1	Test Purpose and Environment	3058

A.11.2.2	Test Requirements	3060

A.12	3060

A.12.1	V2X UE Transmission Timing Accuracy Test	3060

A.12.1.1	V2X UE Transmission Timing Accuracy Test for eNB as Timing Reference	3060

A.12.1.1.1	Test Purpose and Environment	3060

A.12.1.1.2	Test requirements	3061

A.12.1.2	V2X UE Transmission Timing Accuracy Test for SyncRef UE as Timing Reference	3062

A.12.1.2.1	Test Purpose and Environment	3062

A.12.1.2.2	Test Requirements	3062

A.12.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication	3063

A.12.2.1	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for eNB as Timing Reference	3063

A.12.2.1.1	Test Purpose and Environment	3063

A.12.2.1.2	Test Requirements	3064

A.12.2.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for SyncRef UE as Timing Reference	3065

A.12.2.2.1	Test Purpose and Environment	3065

A.12.2.2.2	Test Requirements	3066

A.12.3	V2X Synchronization Reference Selection/Reselection Tests	3066

A.12.3.1	V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority	3066

A.12.3.1.1	Test Purpose and Environment	3066

A.12.3.1.2	Test Requirements	3069

A.12.3.2	V2X Synchronization Reference Selection/Reselection Tests for eNB configured as the highest priority	3070

A.12.3.2.1	Test Purpose and Environment	3070

A.12.3.1.2	Test Requirements	3072

A.12.4	Congestion Control Measurement Test for V2X UE	3072

A.12.4.1	Test Purpose and Environment	3072

A.12.4.2	Test Requirements	3074

A.12.5	Interruptions due to V2X Sidelink Communication	3074

A.12.5.1	Test Purpose and Environment	3074

A.12.5.2	Test Requirements	3076

A.12.6	V2X UE Autonomous Resource Selection/Reselection Measurement Test	3076

A.12.6.1	V2X UE Autonomous Resource Selection/Reselection Tests for PSSCH-RSRP measurements	3076

A.12.6.1.1	Test Purpose and Environment	3076

A.12.6.1.2	Test Requirements	3078

A.12.6.2	V2X UE Autonomous Resource Selection/Reselection Tests for S-RSSI measurements	3078

A.12.6.2.1	Test Purpose and Environment	3078

A.12.6.1.2	Test Requirements	3080

A.12.7	V2X Synchronization Reference Selection/Reselection Tests for V2X Carrier Aggregation	3080

A.12.7.1	Test Purpose and Environment	3080

A.12.7.2	Test Requirements	3083

A.12.8	Interruptions due to V2X Carrier Aggregation	3084

A.12.8.1	Interruptions on a FDD PCell	3084

A.12.8.1.1	Test Purpose and Environment	3084

A.12.8.1.2	Test Requirements	3085

A.12.8.2	Interruptions on a TDD PCell	3085

A.12.8.2.1	Test Purpose and Environment	3085

A.12.8.2.2	Test Requirements	3087

Annex B (normative): Conditions for RRM requirements applicability for operating bands	3088

B.1	Conditions for E-UTRAN RRC_IDLE state mobility	3088

B.1.1	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection	3088

B.1.2	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection	3088

B.1.3	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category M1	3088

B.1.4	Conditions for measurements of intra-frequency NB-IoT cells for cell re-selection for UE Category NB1	3090

B.1.5	Conditions for measurements of inter-frequency NB-IoT cells for cell re-selection for UE Category NB1	3091

B.1.6	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category 1bis	3091

B.1.7	Conditions for measurements of E-UTRAN cells for cell re-selection for UE Category M2	3091

B.1.7.1	Conditions for measurements of intra-frequence E-UTRAN cells for cell selection	3091

B.1.7.2	Condition for measurements of inter-frequence E-UTRAN cells for cell selection	3093

B.1.8	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection for UE Category M1	3094

B.2	Conditions for UE Measurements Procedures in RRC_CONNECTED State	3094

B.2.1	Conditions for E-UTRAN intra-frequency measurements	3094

B.2.2	Conditions for E-UTRAN intra-frequency measurements with autonomous gaps	3095

B.2.3	Conditions for E-UTRAN inter-frequency measurements	3095

B.2.4	Conditions for E-UTRAN inter-frequency measurements with autonomous gaps	3096

B.2.5	Conditions for E-UTRAN OTDOA intra-frequency RSTD Measurements	3096

B.2.6	Conditions for E-UTRAN OTDOA inter-frequency RSTD Measurements	3096

B.2.7	Conditions for Measurements of the secondary component carrier with deactivated SCell	3097

B.2.8	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction	3097

B.2.9	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3098

B.2.10	Conditions for E-UTRAN intra-frequency discovery signal measurements	3098

B.2.10.1	Conditions for E-UTRAN intra-frequency CRS-based measurements	3098

B.2.10.2	Conditions for E-UTRAN intra-frequency CSI-RS based measurements	3098

B.2.11	Conditions for E-UTRAN inter-frequency discovery signal measurements	3099

B.2.11.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	3099

B.2.11.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	3099

B.2.12	Conditions for E-UTRAN intra-frequency discovery signal measurements under operation with frame structure 3	3100

B.2.13	Conditions for E-UTRAN inter-frequency discovery signal measurements under operation with frame structure 3	3100

B.2.13.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	3100

B.2.13.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	3101

B.2.14	Conditions for E-UTRAN intra-frequency measurements by UE Category M1	3101

B.2.15	Conditions for NB-IoT intra-frequency measurements by UE Category NB1	3102

B.2.16	Conditions for NB-IoT intra-frequency RSTD measurements by UE Category NB1	3103

B.2.17	Conditions for NB-IoT inter-frequency RSTD measurements by UE Category NB1	3104

B.2.18	Conditions for E-UTRAN inter-frequency measurements by UE Category M1	3105

B.2.19	Conditions for E-UTRAN measurements by UE Category M2	3106

B.2.19.1	Conditions for E-UTRAN intra-frequency measurements	3106

B.2.19.2	Conditions for E-UTRAN inter-frequency measurements	3106

B.2.20	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M1	3106

B.2.21	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M2	3107

B.2.22	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M1	3107

B.2.23	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M2	3109

B.3	Conditions for measurements performance requirements for UE	3109

B.3.1	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements	3109

B.3.2	Void	3109

B.3.3	Conditions for inter-frequency RSRP and RSRQ Accuracy Requirements	3109

B.3.4	Conditions for inter-frequency relative RSRP and RSRQ Accuracy Requirements	3109

B.3.5	Conditions for UE Rx – Tx time difference	3109

B.3.6	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements	3110

B.3.7	Conditions for inter-frequency RSTD measurements	3110

B.3.8	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements	3110

B.3.9	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction	3110

B.3.10	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction	3110

B.3.11	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3111

B.3.12	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3111

B.3.13	Conditions for UE Rx–Tx Time Difference Measurement under Time Domain Measurement Resource Restriction with CRS Assistance Information	3111

B.3.14	Conditions for Intra-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	3111

B.3.14.1	Conditions for Intra-frequency CRS-based measurements	3111

B.3.14.2	Conditions for Intra-frequency CSI-RS-based measurements	3111

B.3.15	Conditions for Intra-Frequency Relative Discovery Signal Measurement Accuracy Requirements	3112

B.3.15.1	Conditions for Intra-frequency CRS-based measurements	3112

B.3.15.2	Conditions for Intra-frequency CSI-RS-based measurements	3112

B.3.16	Conditions for Inter-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	3113

B.3.16.1	Conditions for Inter-frequency CRS-based measurements	3113

B.3.16.2	Conditions for Inter-frequency CSI-RS-based measurements	3113

B.3.17	Conditions for Inter-Frequency Relative Discovery Signal Measurement Accuracy Requirements	3113

B.3.17.1	Conditions for Inter-frequency CRS-based measurements	3113

B.3.17.2	Conditions for Inter-frequency CSI-RS-based measurements	3113

B.3.18	Conditions for Intra-frequency Absolute RS-SINR Accuracy Requirements	3113

B.3.19	Conditions for Inter-frequency Absolute RS-SINR Accuracy Requirements	3113

B.3.20	Conditions for Inter-frequency Relative RS-SINR Accuracy Requirements	3113

B.3.21	Conditions for Intra-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	3114

B.3.21.1	Conditions for RSRP measurements	3114

B.3.21.2	Conditions for RSRQ measurements	3114

B.3.21.3	Conditions for CSI-RSRP measurements	3114

B.3.22	Conditions for Intra-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	3114

B.3.22.1	Conditions for RSRP measurements	3114

B.3.22.2	Void	3115

B.3.22.3	Conditions for CSI-RSRP measurements	3115

B.3.23	Conditions for Inter-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	3115

B.3.23.1	Conditions for RSRP measurements	3115

B.3.23.2	Conditions for RSRQ measurements	3115

B.3.23.3	Conditions for CSI-RSRP measurements	3115

B.3.24	Conditions for Inter-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	3116

B.3.24.1	Conditions for RSRP measurements	3116

B.3.24.2	Conditions for RSRQ measurements	3116

B.3.24.3	Conditions for CSI-RSRP measurements	3116

B.3.25	Conditions for NB-IoT intra-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	3116

B.3.26	Conditions for NB-IoT inter-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	3116

B.3.27	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements for Category 0	3116

B.3.28	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements for Category 0	3117

B.3.29	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for NB1	3117

B.3.30	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for NB1	3117

B.3.31	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	3117

B.3.32	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	3117

B.3.33	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	3118

B.3.34	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	3118

B.4	RRM Requirements Exceptions	3118

B.4.1	General	3118

B.4.2	Receiver sensitivity relaxation for UE supporting CA	3118

B.4.3	Receiver sensitivity relaxation for UE configured with CA	3118

B.4.3.1	Inter-band carrier aggregation	3118

B.4.3.2	Intra-band non-contiguous carrier aggregation	3118

B.4.3.3	Inter-band carrier aggregation with operating bands without uplink band	3119

B.5	Conditions for Measurement Performance Requirements for ProSe UE	3119

B.5.1	Conditions for S-RSRP Accuracy Requirements	3119

B.5.2	Conditions for Relative S-RSRP Accuracy Requirements	3119

B.5.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	3120

B.5.4	Conditions for SD-RSRP Accuracy Requirements	3120

B.5.5	Conditions for Relative SD-RSRP Accuracy Requirements	3121

B.6	Conditions for V2X	3121

B.6.1	Test parameters for GNSS signals	3121

B.6.2	Conditions for Absolute S-RSRP Accuracy Requirements	3121

B.6.3	Conditions for Relative S-RSRP Accuracy Requirements	3121

B.6.4	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	3122

B.6.5	Conditions for Absolute PSSCH-RSRP Accuracy Requirements	3122

B.7	Conditions for sTTI and 1ms-TTI with 3 Subframe HARQ Processing	3122

B.7.1	Conditions for Maximum Timing Difference Between Uplink and Downlink Carriers in Carrier Aggregation	3122

Annex C (informative): Change history:	3124