3GPP TS 36.508 V16.8.0 (2021-03)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC);

Common test environments for User Equipment (UE) conformance testing

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.	
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

mobile, UE, terminal, testing, LTE

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	28

Introduction	28

1	Scope	29

2	References	29

3	Definitions, symbols and abbreviations	32

3.1	Definitions	32

3.2	Symbols	32

3.3	Abbreviations	32

4	Common test environment	33

4.1	Environmental conditions	33

4.1.1	Temperature	33

4.1.2	Voltage	33

4.2	Common requirements of test equipment	34

4.2.1	General functional requirements	34

4.2.2	Minimum functional requirements	35

4.2.2.1	Supported Cell Configuration	35

4.2.2.1.1	Supported Channels	35

4.2.2.2	Support of Tcell timing offset	36

4.2.2.3	Supported Sidelink Configuration	36

4.2.2.3.1	Supported Sidelink Channels	36

4.3	Reference test conditions	37

4.3.1	Test frequencies	37

4.3.1.1	FDD Mode Test frequencies	40

4.3.1.1.1	FDD reference test frequencies for operating band 1	40

4.3.1.1.1A	FDD reference test frequencies for CA in operating band 1	41

4.3.1.1.2	FDD reference test frequencies for operating band 2	41

4.3.1.1.2A	FDD reference test frequencies for CA in operating band 2	42

4.3.1.1.3	FDD reference test frequencies for operating band 3	44

4.3.1.1.3A	FDD reference test frequencies for CA in operating band 3	44

4.3.1.1.4	FDD reference test frequencies for operating band 4	45

4.3.1.1.4A	FDD reference test frequencies for CA in operating band 4	46

4.3.1.1.5	FDD reference test frequencies for operating band 5	47

4.3.1.1.5A	FDD reference test frequencies for CA in operating band 5	47

4.3.1.1.6	FDD reference test frequencies for operating band 6	48

4.3.1.1.7	FDD reference test frequencies for operating band 7	48

4.3.1.1.7A	FDD reference test frequencies for CA in operating band 7	49

4.3.1.1.8	FDD reference test frequencies for operating band 8	50

4.3.1.1.8A	FDD reference test frequencies for CA in operating band 8	51

4.3.1.1.9	FDD reference test frequencies for operating band 9	51

4.3.1.1.10	FDD reference test frequencies for operating band 10	51

4.3.1.1.11	FDD reference test frequencies for operating band 11	52

4.3.1.1.12	FDD reference test frequencies for operating band 12	52

4.3.1.1.12A	FDD reference test frequencies for CA in operating band 12	52

4.3.1.1.13	FDD reference test frequencies for operating band 13	53

4.3.1.1.14	FDD reference test frequencies for operating band 14	53

4.3.1.1.15	FDD reference test frequencies for operating band 15	53

4.3.1.1.16	FDD reference test frequencies for operating band 16	53

4.3.1.1.17	FDD reference test frequencies for operating band 17	54

4.3.1.1.18	FDD reference test frequencies for operating band 18	54

4.3.1.1.19	FDD reference test frequencies for operating band 19	54

4.3.1.1.20	FDD reference test frequencies for operating band 20	54

4.3.1.1.21	FDD reference test frequencies for operating band 21	55

4.3.1.1.22	FDD reference test frequencies for operating band 22	55

4.3.1.1.23	FDD reference test frequencies for operating band 23	55

4.3.1.1.23A	FDD reference test frequencies for CA in operating band 23	56

4.3.1.1.24	FDD reference test frequencies for operating band 24	56

4.3.1.1.25	FDD reference test frequencies for operating band 25	57

4.3.1.1.25A	FDD reference test frequencies for CA in operating band 25	57

4.3.1.1.26	FDD reference test frequencies for operating band 26	58

4.3.1.1.27	FDD reference test frequencies for operating band 27	58

4.3.1.1.27A	FDD reference test frequencies for CA in operating band 27	59

4.3.1.1.28	FDD reference test frequencies for operating band 28	59

4.3.1.1.29	FDD reference test frequencies for CA in operating band 29	60

4.3.1.1.31	FDD reference test frequencies for operating band 31	61

4.3.1.1.32	FDD reference test frequencies for CA in operating band 32	61

4.3.1.1.33 to 4.3.1.1.64	Void	61

4.3.1.1.65	FDD reference test frequencies for operating band 65	61

4.3.1.1.66	FDD reference test frequencies for operating band 66	62

4.3.1.1.66A	FDD reference test frequencies for CA in operating band 66	63

4.3.1.1.67	FDD reference test frequencies for CA in operating band 67	76

4.3.1.1.68	FDD reference test frequencies for operating band 68	77

4.3.1.1.69	FDD reference test frequencies for operating band 69	77

4.3.1.1.70	FDD reference test frequencies for operating band 70	77

4.3.1.1.70A	FDD reference test frequencies for CA in operating band 70	78

4.3.1.1.71	FDD reference test frequencies for operating band 71	78

4.3.1.1.72	FDD reference test frequencies for operating band 72	78

4.3.1.1.73	FDD reference test frequencies for operating band 73	79

4.3.1.1.74	FDD reference test frequencies for operating band 74	79

4.3.1.1.75 to 4.3.1.1.84 FFS	79

4.3.1.1.85	FDD reference test frequencies for operating band 85	79

4.3.1.2	TDD Mode Test frequencies	80

4.3.1.2.1	TDD reference test frequencies for Operating Band 33	80

4.3.1.2.2	TDD reference test frequencies for Operating Band 34	80

4.3.1.2.3	TDD reference test frequencies for Operating Band 35	80

4.3.1.2.4	TDD reference test frequencies for Operating Band 36	81

4.3.1.2.5	TDD reference test frequencies for Operating Band 37	81

4.3.1.2.6	TDD reference test frequencies for Operating Band 38	81

4.3.1.2.6A	TDD reference test frequencies for CA in operating band 38	82

4.3.1.2.7	TDD reference test frequencies for Operating Band 39	82

4.3.1.2.7A	TDD reference test frequencies for CA in Operating Band 39	83

4.3.1.2.8	TDD reference test frequencies for Operating Band 40	83

4.3.1.2.8A	TDD reference test frequencies for CA in operating band 40	84

4.3.1.2.9	TDD reference test frequencies for Operating Band 41	90

4.3.1.2.9A	TDD reference test frequencies for CA in operating band 41	91

4.3.1.2.10	TDD reference test frequencies for Operating Band 42	104

4.3.1.2.10A	TDD reference test frequencies for CA in operating band 42	104

4.3.1.2.11	TDD reference test frequencies for Operating Band 43	114

4.3.1.2.12	TDD reference test frequencies for Operating Band 44	114

4.3.1.2.13	TDD reference test frequencies for Operating Band 45	114

4.3.1.2.14	TDD reference test frequencies for Operating Band 46	115

4.3.1.2.14A	TDD reference test frequencies for CA in operating Band 46	115

4.3.1.2.15	TDD reference test frequencies for Operating Band 47	116

4.3.1.2.16	TDD reference test frequencies for Operating Band 48	117

4.3.1.2.16A	TDD reference test frequencies for CA in operating band 48	117

4.3.1.2.17	TDD reference test frequencies for Operating Band 53	119

4.3.1.3	HRPD Test frequencies	119

4.3.1.3.1	HRPD test frequencies for Band Class 0	119

4.3.1.3.2	HRPD test frequencies for Band Class 1	119

4.3.1.3.3	HRPD test frequencies for Band Class 3	119

4.3.1.3.4	HRPD test frequencies for Band Class 4	119

4.3.1.3.5	HRPD test frequencies for Band Class 6	120

4.3.1.3.6	HRPD test frequencies for Band Class 10	120

4.3.1.3.7	HRPD test frequencies for Band Class 15	120

4.3.1.4	1xRTT Test frequencies	120

4.3.1.4.1	1xRTT test frequencies for Band Class 0	120

4.3.1.4.2	1xRTT test frequencies for Band Class 1	120

4.3.1.4.3	1xRTT test frequencies for Band Class 3	121

4.3.1.4.4	1xRTT test frequencies for Band Class 4	121

4.3.1.4.5	1xRTT test frequencies for Band Class 6	121

4.3.1.4.6	1xRTT test frequencies for Band Class 10	121

4.3.1.4.7	1xRTT test frequencies for Band Class 15	121

4.3.1.5	MFBI Test frequencies	121

4.3.1.5.1	MFBI Test frequencies for operation band 2 overlapping with band 25	122

4.3.1.5.2	MFBI Test frequencies for operation band 3 overlapping with band 9	122

4.3.1.5.3	MFBI Test frequencies for operation band 4 overlapping with band 10	122

4.3.1.5.4	MFBI Test frequencies for operation band 5 overlapping with band 18	122

4.3.1.5.5	MFBI Test frequencies for operation band 5 overlapping with band 19	123

4.3.1.5.6	MFBI Test frequencies for operation band 5 overlapping with band 26	123

4.3.1.5.7	MFBI Test frequencies for operation band 9 overlapping with band 3	123

4.3.1.5.8	MFBI Test frequencies for operation band 10 overlapping with band 4	123

4.3.1.5.9	MFBI Test frequencies for operation band 12 overlapping with band 17	123

4.3.1.5.10	MFBI Test frequencies for operation band 17 overlapping with band 12	124

4.3.1.5.11	MFBI Test frequencies for operation band 18 overlapping with band 5	124

4.3.1.5.12	MFBI Test frequencies for operation band 18 overlapping with band 26	124

4.3.1.5.13	MFBI Test frequencies for operation band 18 overlapping with band 27	124

4.3.1.5.14	MFBI Test frequencies for operation band 19 overlapping with band 5	124

4.3.1.5.15	MFBI Test frequencies for operation band 19 overlapping with band 26	124

4.3.1.5.16	MFBI Test frequencies for operation band 25 overlapping with band 2	125

4.3.1.5.17	MFBI Test frequencies for operation band 26 overlapping with band 5	125

4.3.1.5.18	MFBI Test frequencies for operation band 26 overlapping with band 18	125

4.3.1.5.19	MFBI Test frequencies for operation band 26 overlapping with band 19	126

4.3.1.5.20	MFBI Test frequencies for operation band 26 overlapping with band 27	126

4.3.1.5.21	MFBI Test frequencies for operation band 27 overlapping with band 18	127

4.3.1.5.22	MFBI Test frequencies for operation band 27 overlapping with band 26	127

4.3.1.5.23	MFBI Test frequencies for operation band 33 overlapping with band 39	127

4.3.1.5.24	MFBI Test frequencies for operation band 38 overlapping with band 41	127

4.3.1.5.25	MFBI Test frequencies for operation band 39 overlapping with band 33	127

4.3.1.5.26	MFBI Test frequencies for operation band 41 overlapping with band 38	128

4.3.1.5.27	MFBI Test frequencies for operation band 66 overlapping with band 4	128

4.3.1.5.28	MFBI Test frequencies for operation band 66 overlapping with band 10	128

4.3.1.6	WLAN Test frequencies	129

4.3.1.6.1	WLAN Test frequencies for 2.4 GHz ISM Band	129

4.3.1.6.2	WLAN Test frequencies for 5 GHz ISM Band	129

4.3.1.7	Bluetooth Test frequencies	129

4.3.1.7.1	Bluetooth Test frequencies for 2.4 GHz ISM Band	129

4.3.2	Radio conditions	129

4.3.2.1	Normal propagation condition	129

4.3.3	Physical channel allocations	129

4.3.3.1	Antennas	129

4.3.3.2	Downlink physical channels and physical signals	130

4.3.3.3	Mapping of downlink physical channels and signals to physical resources	130

4.3.3.4	Uplink physical channels and physical signals	135

4.3.3.5	Mapping of uplink physical channels and signals to physical resources	135

4.3.4	Signal levels	135

4.3.4.1	Downlink signal levels	135

4.3.4.2	Uplink signal levels	135

4.3.5	Standard test signals	136

4.3.5.1	Downlink test signals	136

4.3.5.2	Uplink test signals	136

4.3.6	Physical layer parameters	136

4.3.6.1	Downlink physical layer parameters	136

4.3.6.1.1	Physical layer parameters for DCI format 0	136

4.3.6.1.1A	Physical layer parameters for DCI format 0C	137

4.3.6.1.1B	Physical layer parameters for DCI format 0A	138

4.3.6.1.1C	Physical layer parameters for DCI format 0B	138

4.3.6.1.2	Physical layer parameters for DCI format 1	139

4.3.6.1.3	Physical layer parameters for DCI format 1A	139

4.3.6.1.3A	Physical layer parameters for DCI format 1B	140

4.3.6.1.4	Physical layer parameters for DCI format 1C	140

4.3.6.1.5	Physical layer parameters for DCI format 2	141

4.3.6.1.6	Physical layer parameters for DCI format 2A	142

4.3.6.1.6A	Physical layer parameters for DCI format 3B	142

4.3.6.1.6B	Physical layer parameters for DCI format 4A	143

4.3.6.1.6C	Physical layer parameters for DCI format 4B	143

4.3.6.1.7	Physical layer parameters for DCI format 5	144

4.3.6.1.7A	Physical layer parameters for DCI format 5A	144

4.3.6.1.8	Physical layer parameters for DCI format 6-0A	144

4.3.6.1.9	Physical layer parameters for DCI format 6-0B	146

4.3.6.1.10	Physical layer parameters for DCI format 6-1A	147

4.3.6.1.11	Physical layer parameters for DCI format 6-1B	148

4.3.6.1.12	Physical layer parameters for DCI format 6-2	149

4.3.6.1.13	Physical layer parameters for DCI format 7-0A	150

4.3.6.1.14	Physical layer parameters for DCI format 7-0B	151

4.3.6.1.15	Physical layer parameters for DCI format 7-1A	152

4.3.6.1.16	Physical layer parameters for DCI format 7-1B	153

4.3.6.1.17	Physical layer parameters for DCI format 7-1C	154

4.3.6.1.18	Physical layer parameters for DCI format 7-1D	155

4.3.6.1.19	Physical layer parameters for DCI format 7-1E	156

4.3.6.1.20	Physical layer parameters for DCI format 7-1F	157

4.3.6.1.21	Physical layer parameters for DCI format 7-1G	158

4.4	Reference system configurations	158

4.4.1	Simulated network scenarios	158

4.4.1.1	Single cell network scenarios	158

4.4.1.2	E-UTRA single mode multi cell network scenarios	158

4.4.1.3	E-UTRA dual mode multi cell network scenarios	159

4.4.1.4	3GPP Inter-RAT network scenarios	159

4.4.1.5	3GPP2 Inter-RAT network scenarios	159

4.4.1.6	WLAN Inter-RAT network scenarios	159

4.4.1.7	Bluetooth Inter-RAT network scenarios	159

4.4.2	Simulated cells	159

4.4.3	Common parameters for simulated E-UTRA cells	164

4.4.3.1	Common configurations of system information blocks	164

4.4.3.1.1	Combinations of system information blocks	164

4.4.3.1.2	Scheduling of system information blocks	169

4.4.3.2	Common contents of system information messages	174

-	MasterInformationBlock	174

-	SystemInformation	175

-	SystemInformation-BR-r13	175

-	SystemInformationBlockType1	177

-	SystemInformationBlockType1-BR-r13	179

4.4.3.3	Common contents of system information blocks	182

-	SystemInformationBlockType2	182

-	SystemInformationBlockType3	184

-	SystemInformationBlockType4	185

-	SystemInformationBlockType5	185

-	SystemInformationBlockType6	188

-	SystemInformationBlockType7	189

-	SystemInformationBlockType8	190

-	SystemInformationBlockType9	192

-	SystemInformationBlockType10	192

-	SystemInformationBlockType11	194

-	SystemInformationBlockType12	196

-	SystemInformationBlockType13	197

-	SystemInformationBlockType14	197

-	SystemInformationBlockType15	198

-	SystemInformationBlockType17	199

-	SystemInformationBlockType18	199

-	SystemInformationBlockType19	205

-	SystemInformationBlockType20	209

-	SystemInformationBlockType21	209

-	SystemInformationBlockType24	210

4.4.3.4	Channel-bandwidth-dependent parameters in system information blocks	212

4.4.4	Common parameters for simulated UTRA cells	213

4.4.4.1	Common contents of system information blocks for UTRA cells	213

-	System Information Block type 19	213

4.4.4.2	UTRA SIB scheduling for inter EUTRA - UTRA test	216

4.4.4.3	UTRA SIB scheduling for inter EUTRA – UTRA - GERAN test	216

4.4.5	Common parameters for simulated GERAN cells	217

4.4.6	Common parameters for simulated CDMA2000 cells	219

4.4.7	Default parameters specific for simulated cells	220

4.4.7.1	Common contents of HRPD Overhead messages	220

4.4.7.2	Common contents of 1XRTT Overhead messages	224

4.4.7.2.1	Configuration sequence number	224

4.4.7.2.2	Over Head messages	225

4.4.8	Common parameters for simulated WLAN AP’s	232

4.4.9	Common parameters for simulated Bluetooth beacon’s	233

4.5	Generic procedures	234

4.5.1	UE test states	234

4.5.2	UE Registration (State 2)	239

4.5.2.1	Initial conditions	240

4.5.2.2	Definition of system information messages	240

4.5.2.3	Procedure	241

4.5.2.4	Specific message contents	243

4.5.2A	UE Registration, UE Test Mode Activated (State 2A)	246

4.5.2A.1	Initial conditions	246

4.5.2A.2	Definition of system information messages	246

4.5.2A.3	Procedure	247

4.5.2A.4	Specific message contents	249

4.5.2AA	UE Registration in cell supporting BL/CE UE (State 2-CE)	249

4.5.2AA.1	Initial conditions	249

4.5.2AA.2	Definition of system information messages	250

4.5.2AA.3	Procedure	250

4.5.2AA.4	Specific message contents	250

4.5.2AB	UE Registration, UE Test Mode Activated in cell supporting BL/CE UE (State 2A-CE)	250

4.5.2AB.1	Initial conditions	250

4.5.2AB.2	Definition of system information messages	250

4.5.2AB.3	Procedure	251

4.5.2AB.4	Specific message contents	251

4.5.2B	UE Registration, pre-registration on HRPD (State 2B)	251

4.5.2B.1	Initial conditions	251

4.5.2B.2	Definition of system information messages	251

4.5.2B.3	Procedure	252

4.5.2B.4	Specific message contents	256

4.5.2C	UE Registration, pre-registration on 1xRTT (State 2C)	257

4.5.2C.1	Initial conditions	257

4.5.2C.2	Definition of system information messages	257

4.5.2C.3	Procedure	259

4.5.2C.4	Specific message contents	259

4.5.2D	UE Registration, 2 PDN for RAN Assisted WLAN Interworking (State 2)	264

4.5.2D.1	Initial conditions	264

4.5.2D.2	Definition of system information messages	264

4.5.2D.3	Procedure	265

4.5.2D.4	Specific message contents	265

4.5.3	Generic Radio Bearer Establishment (State 3)	267

4.5.3.1	Initial conditions	267

4.5.3.2	Definition of system information messages	267

4.5.3.3	Procedure	268

4.5.3.4	Specific message contents	269

4.5.3A	Generic Radio Bearer Establishment, UE Test Mode Activated (State 3A)	269

4.5.3A.1	Initial conditions	269

4.5.3A.2	Definition of system information messages	269

4.5.3A.3	Procedure	270

4.5.3A.4	Specific message contents	270

4.5.3AA	Generic Radio Bearer Establishment (State 3-CE)	270

4.5.3AA.1	Initial conditions	270

4.5.3AA.2	Definition of system information messages	270

4.5.3AA.3	Procedure	270

4.5.3AA.4	Specific message contents	270

4.5.3AB	Generic Radio Bearer Establishment, UE Test Mode Activated (State 3A-CE)	270

4.5.3AB.1	Initial conditions	270

4.5.3AB.2	Definition of system information messages	271

4.5.3AB.3	Procedure	271

4.5.3AB.4	Specific message contents	271

4.5.3B	Generic Radio Bearer Establishment, pre-registered on HRPD (State 3B)	271

4.5.3B.1	Initial conditions	271

4.5.3B.2	Definition of system information messages	271

4.5.3B.3	Procedure	271

4.5.3B.4	Specific message contents	271

4.5.3C	Generic Radio Bearer Establishment, pre-registered on 1xRTT (State 3C)	271

4.5.3C.1	Initial conditions	271

4.5.3C.2	Definition of system information messages	272

4.5.3C.3	Procedure	272

4.5.3C.4	Specific message contents	272

4.5.3D	Generic Radio Bearer Establishment for RAN Assisted WLAN Interworking (State 3)	272

4.5.3D.1	Initial conditions	272

4.5.3D.2	Definition of system information messages	272

4.5.3D.3	Procedure	273

4.5.3D.4	Specific message contents	273

4.5.3E	Control plane CIoT connection request (State 3-CP)	273

4.5.3E.1	Initial conditions	273

4.5.3E.2	Definition of system information messages	273

4.5.3E.3	Procedure	274

4.5.3E.4	Specific message contents	274

4.5.3EA	Control plane CIoT connection request, UE Test Mode Activated (State 3A-CP)	274

4.5.3EA.1	Initial conditions	274

4.5.3EA.2	Definition of system information messages	274

4.5.3EA.3	Procedure	274

4.5.3EA.4	Specific message contents	274

4.5.3F	User plane CIoT connection request (State 3-UP)	275

4.5.3F.1	Initial conditions	275

4.5.3F.2	Definition of system information messages	275

4.5.3F.3	Procedure	275

4.5.3F.4	Specific message contents	275

4.5.3FA	User plane CIoT connection request, UE Test Mode Activated (State 3A-UP)	275

4.5.3FA.1	Initial conditions	275

4.5.3FA.2	Definition of system information messages	276

4.5.3FA.3	Procedure	276

4.5.3FA.4	Specific message contents	276

4.5.4	Loopback Activation (State 4)	276

4.5.4.1	Initial conditions	276

4.5.4.2	Definition of system information messages	276

4.5.4.3	Procedure	276

4.5.4.4	Specific message contents	276

4.5.4A	Loopback Activation in cell supporting BL/CE UE (State 4-CE)	276

4.5.4A.1	Initial conditions	276

4.5.4A.2	Definition of system information messages	277

4.5.4A.3	Procedure	277

4.5.4A.4	Specific message contents	277

4.5.4B	Loopback Activation user plane (State 4A-UP)	277

4.5.4B.1	Initial conditions	277

4.5.4B.2	Definition of system information messages	277

4.5.4B.3	Procedure	277

4.5.4B.4	Specific message contents	277

4.5.5	HRPD registration (State H2)	278

4.5.5.1	Initial conditions	278

4.5.5.2	Definition of system information messages	278

4.5.5.3	Procedure	278

4.5.5.4	Specific message contents	278

4.5.5A	HRPD registration, pre-registration on E-UTRAN (State H2A)	278

4.5.5A.1	Initial conditions	278

4.5.5A.2	Definition of system information messages	278

4.5.5A.3	Procedure	278

4.5.5A.4	Specific message contents	278

4.5.6	HRPD session establishment (State H3)	279

4.5.6.1	Initial conditions	279

4.5.6.2	Definition of system information messages	279

4.5.6.3	Procedure	279

4.5.6.4	Specific message contents	279

4.5.6A	HRPD session establishment, pre-registered on E-UTRAN (State H3A)	279

4.5.6A.1	Initial conditions	279

4.5.6A.2	Definition of system information messages	279

4.5.6A.3	Procedure	279

4.5.6A.4	Specific message contents	279

4.5.7	Out of Coverage (State 5)	280

4.5.7.1	Initial conditions	280

4.5.7.2	Definition of system information messages	280

4.5.7.3	Procedure	280

4.5.8	Out of Coverage, V2X setup (State 5-V2X)	280

4.5.8.1	Initial conditions	280

4.5.8.2	Definition of system information messages	280

4.5.8.3	Procedure	280

4.5.9	Out of Coverage, Test Loopback Activation, V2X setup (State 5A-V2X)	280

4.5.9.1	Initial conditions	280

4.5.9.2	Definition of system information messages	281

4.5.9.3	Procedure	281

4.5.9.4	Specific message contents	281

4.5A	Other generic procedures	282

4.5A.1	Procedure for IP address allocation in the U-plane	282

4.5A.2	Tracking area updating procedure	282

4.5A.3	Procedure for IMS signalling	283

4.5A.3.1	Specific message contents	284

4.5A.3A	Procedure for IMS Signalling over UTRA	284

4.5A.3A.1	Initial conditions	284

4.5A.3A.2	Procedure	285

4.5A.3A.3	Specific message contents	286

4.5A.3B	Procedure for preventing IMS Signalling over GERAN	288

4.5A.3B.1	Initial conditions	288

4.5A.3B.2	Procedure	289

4.5A.3B.3	Specific message contents	289

4.5A.4	Generic Test Procedure for IMS Emergency call establishment in EUTRA: Normal Service	290

4.5A.4.1	Initial conditions	290

4.5A.4.2	Definition of system information messages	290

4.5A.4.3	Procedure	290

4.5A.4.4	Specific message contents	292

4.5A.5	Generic Test Procedure for IMS Emergency call establishment in EUTRA: Limited Service	293

4.5A.5.1	Initial conditions	293

4.5A.5.2	Definition of system information messages	293

4.5A.5.3	Procedure	294

4.5A.5.4	Specific message contents	296

4.5A.6	Generic Test Procedure for IMS MO speech call establishment in E-UTRA	298

4.5A.6.1	Initial conditions	298

4.5A.6.2	Definition of system information messages	298

4.5A.6.3	Procedure	299

4.5A.6.4	Specific message contents	300

4.5A.7	Generic Test Procedure for IMS MT Speech call establishment in E-UTRA	300

4.5A.7.1	Initial conditions	300

4.5A.7.2	Definition of system information messages	300

4.5A.7.3	Procedure	301

4.5A.7.4	Specific message contents	302

4.5A.8	Generic Test Procedure for IMS MO video call establishment in E-UTRA	302

4.5A.8.1	Initial conditions	302

4.5A.8.2	Definition of system information messages	302

4.5A.8.3	Procedure	303

4.5A.8.4	Specific message contents	304

4.5A.9	Generic Test Procedure for IMS MT video call establishment in E-UTRA	304

4.5A.9.1	Initial conditions	304

4.5A.9.2	Definition of system information messages	304

4.5A.9.3	Procedure	305

4.5A.9.4	Specific message contents	305

4.5A.10	Generic Test Procedure for IMS MO speech and aSRVCC in E-UTRA	306

4.5A.10.1	Initial conditions	306

4.5A.10.2	Definition of system information messages	306

4.5A.10.3	Procedure	307

4.5A.10.4	Specific message contents	308

4.5A.11	Generic Test Procedure for IMS MO add video establishment in E-UTRA	308

4.5A.11.1	Initial conditions	308

4.5A.11.2	Definition of system information messages	308

4.5A.11.3	Procedure	308

4.5A.11.4	Specific message contents	309

4.5A.12	Generic Test Procedure for IMS MT add video establishment in E-UTRA	309

4.5A.12.1	Initial conditions	309

4.5A.12.2	Definition of system information messages	309

4.5A.12.3	Procedure	309

4.5A.12.4	Specific message contents	310

4.5A.14	Generic Test Procedure for IMS XCAP establishment in EUTRA	310

4.5A.14.1	Initial conditions	310

4.5A.14.2	Definition of system information messages	310

4.5A.14.3	Procedure	311

4.5A.14.4	Specific message contents	311

4.5A.15	Generic Test Procedure for EPS Bearer Deactivation	311

4.5A.15.1	Initial conditions	311

4.5A.15.2	Definition of system information messages	311

4.5A.15.3	Procedure	312

4.5A.15.4	Specific message contents	312

4.5A.15A	Generic Test Procedure for User or Network Initiated EPS Bearer Deactivation	313

4.5A.15A.1	Initial conditions	313

4.5A.15A.2	Definition of system information messages	313

4.5A.15A.3	Procedure	313

4.5A.15A.4	Specific message contents	314

4.5A.16	Generic Test Procedure to establish additional PDN connectivity	314

4.5A.16.1	Initial conditions	314

4.5A.16.2	Definition of system information messages	314

4.5A.16.3	Procedure	315

4.5A.16.4	Specific message contents	315

4.5A.17	Generic Test Procedure for user initiated release of additional PDN connectivity	316

4.5A.17.1	Initial conditions	316

4.5A.17.2	Definition of system information messages	316

4.5A.17.3	Procedure	317

4.5A.17.4	Specific message contents	317

4.5A.18	Generic Test Procedure for network initiated release of additional PDN connectivity	318

4.5A.18.1	Initial conditions	318

4.5A.18.2	Definition of system information messages	319

4.5A.18.3	Procedure	319

4.5A.18.4	Specific message contents	319

4.5A.19	Generic Test Procedure for IMS MO speech call establishment in E-UTRA / EVS	320

4.5A.19.1	Initial conditions	320

4.5A.19.2	Definition of system information messages	320

4.5A.19.3	Procedure	320

4.5A.19.4	Specific message contents	321

4.5A.20	Generic Test Procedure for IMS MT speech call establishment in E-UTRA / EVS	321

4.5A.20.1	Initial conditions	321

4.5A.20.2	Definition of system information messages	321

4.5A.20.3	Procedure	321

4.5A.20.4	Specific message contents	321

4.5A.21	Generic Test Procedure for IMS MO Customized Alerting Tones and speech establishment in E-UTRA	322

4.5A.21.1	Initial conditions	322

4.5A.21.2	Definition of system information messages	322

4.5A.21.3	Procedure	322

4.5A.21.4	Specific message contents	322

4.5A.22	Communication with the ProSe Function: Initial Access	322

4.5A.22.1	Initial conditions	322

4.5A.22.2	Definition of system information messages	322

4.5A.22.3	Procedure	323

4.5A.22.4	Specific message contents	325

4.5A.22A	Communication with the ProSe Function: Subsequent Access	325

4.5A.22A.1	Initial conditions	325

4.5A.22A.2	Definition of system information messages	325

4.5A.22A.3	Procedure	325

4.5A.22A.4	Specific message contents	326

4.5A.23	Generic Test Procedure for IMS registration in EPC / WLAN	326

4.5A.23.1	Initial conditions	326

4.5A.23.2	Definition of system information messages	326

4.5A.23.3	Procedure	327

4.5A.23.4	Specific message contents	327

4.5A.23A	Generic Test Procedure for IPsec Tunnel Disconnection in EPC / WLAN	327

4.5A.23A.1	Initial conditions	327

4.5A.23A.2	Definition of system information messages	327

4.5A.23A.3	Procedure	327

4.5A.23A.4	Specific message contents	327

4.5A.24	Generic Test Procedure for IMS emergency call establishment in EPC / WLAN	328

4.5A.24.1	Initial conditions	328

4.5A.24.2	Definition of system information messages	328

4.5A.24.3	Procedure	328

4.5A.24.4	Specific message contents	328

4.5A.25	Generic Test Procedure for XCAP establishment in EPC / WLAN	328

4.5A.25.1	Initial conditions	328

4.5A.25.2	Definition of system information messages	328

4.5A.25.3	Procedure	329

4.5A.25.4	Specific message contents	329

4.5A.26	Generic Test Procedure for eCall over IMS establishment in EUTRA: Normal Service	329

4.5A.26.1	Initial conditions	329

4.5A.26.2	Definition of system information messages	329

4.5A.26.3	Procedure	329

4.5A.26.4	Specific message contents	331

4.5A.27	Generic Test Procedure for eCall over IMS establishment in EUTRA: eCall Only Support	332

4.5A.27.1	Initial conditions	332

4.5A.27.2	Definition of system information messages	332

4.5A.27.3	Procedure	332

4.5A.27.4	Specific message contents	334

4.5A.28	Generic Test Procedure for Converged IP Communications establishment in EUTRA	335

4.5A.28.1	Initial conditions	335

4.5A.28.2	Definition of system information messages	335

4.5A.28.3	Procedure	335

4.5A.28.4	Specific message contents	335

4.5A.29	Generic Test Procedure for Converged IP Communications establishment in EPC / WLAN	336

4.5A.29.1	Initial conditions	336

4.5A.29.2	Definition of system information messages	337

4.5A.29.3	Procedure	337

4.5A.29.4	Specific message contents	337

4.5A.30	Generic Test Procedure for IMS Re-registration in E-UTRAN	337

4.5A.30.1	Initial conditions	337

4.5A.30.2	Definition of system information messages	337

4.5A.30.3	Procedure	338

4.5A.30.4	Specific message contents	338

4.6	Default RRC message and information elements contents	338

4.6.1	Contents of RRC messages	338

–	CounterCheck	338

–	CounterCheckResponse	339

–	CSFBParametersRequestCDMA2000	339

–	CSFBParametersResponseCDMA2000	339

–	DLInformationTransfer	340

–	HandoverFromEUTRAPreparationRequest	340

-	LoggedMeasurementConfiguration	341

-	MasterInformationBlock-SL	342

-	MasterInformationBlock-SL-V2X	343

–	MBMSCountingRequest	343

–	MBMSCountingResponse	344

–	MBMSInterestIndication	344

–	MBSFNAreaConfiguration	345

–	MeasurementReport	345

–	MobilityFromEUTRACommand	346

–	Paging	346

–	RRCConnectionReconfiguration	347

–	RRCConnectionReconfiguration (SideLink)	353

–	RRCConnectionReconfiguration (V2X)	359

–	RRCConnectionReconfigurationComplete	360

–	RRCConnectionReestablishment	361

–	RRCConnectionReestablishmentComplete	361

–	RRCConnectionReestablishmentReject	361

–	RRCConnectionReestablishmentRequest	362

–	RRCConnectionReject	362

–	RRCConnectionRelease	363

–	RRCConnectionRequest	363

–	RRCConnectionResume	364

–	RRCConnectionResumeComplete	364

–	RRCConnectionResumeRequest	365

–	RRCConnectionSetup	365

–	RRCConnectionSetupComplete	366

–	SCGFailureInformationNR	367

–	SCPTMConfiguration	369

–	SCPTMConfiguration-BR	370

–	SecurityModeCommand	371

–	SecurityModeComplete	371

–	SecurityModeFailure	371

–	SidelinkUEInformation	372

–	SidelinkUEInformation (V2X)	373

–	UECapabilityEnquiry	374

–	UECapabilityInformation	375

–	UEInformationRequest	382

–	UEInformationResponse	383

-	ULHandoverPreparationTransfer	383

–	ULInformationTransfer	384

–	UEAssistanceInformation	384

–	ULInformationTransferMRDC	385

4.6.2	System information blocks	385

4.6.3	Radio resource control information elements	385

–	BCCH-Config-DEFAULT	385

–	CellSelectionInfoCE-r13-DEFAULT	386

–	CQI-ReportAperiodic-r10-DEFAULT	386

–	CQI-ReportConfig-DEFAULT	387

–	CQI-ReportConfig-r10-DEFAULT	387

–	CQI-ReportConfig-v1130-eIMTA	391

–	CQI-ReportConfig-v1250-DEFAULT	392

–	CQI-ReportConfigSCell-r10-DEFAULT	393

–	CQI-ReportPeriodic-r10-DEFAULT	393

–	CSI-RS-ConfigNZP-r11-DEFAULT	394

–	CSI-RS-ConfigZP-r11-DEFAULT	394

–	DMRS-Config-r11-DEFAULT	395

–	DRB-ToAddModList-RECONFIG	395

–	EPDCCH-Config-r11-DEFAULT	395

–	EPDCCH-Config-r11-eIMTA	398

–	FreqHoppingParameters-r13-DEFAULT	399

–	PCCH-Config-DEFAULT	400

–	PCCH-Config-v1310-DEFAULT	400

–	PHICH-Config-DEFAULT	400

–	PDSCH-ConfigCommon-DEFAULT	401

–	PDSCH-ConfigCommon-v1310-DEFAULT	401

–	PDSCH-ConfigDedicated-DEFAULT	401

–	PDSCH-ConfigDedicated-v1130-DEFAULT	402

–	PhysicalConfigDedicatedSCell-r10-DEFAULT	403

–	PhysicalConfigDedicatedSCell-r10-eIMTA	405

–	PRACH-Config-DEFAULT	406

-	PRACH-Config-v1310-DEFAULT	407

-	PRACH-ConfigSIB-DEFAULT	409

-	PRACH-ConfigSIB-v1310-DEFAULT	410

-	PRACH-ConfigSIB-v1530-DEFAULT	414

–	PUCCH-ConfigCommon-DEFAULT	416

-	PUCCH-ConfigCommon-v1310-DEFAULT	416

–	PUCCH-ConfigDedicated-DEFAULT	417

–	PUCCH-ConfigDedicated-v1020-DEFAULT	418

–	PUCCH-ConfigDedicated-v1130-DEFAULT	419

–	PUCCH-ConfigDedicated-v1250-DEFAULT	419

–	PUCCH-ConfigDedicated-r13-DEFAULT	420

–	PUSCH-ConfigCommon-DEFAULT	421

–	PUSCH-ConfigCommon-v1310DEFAULT	421

–	PUSCH-ConfigDedicated-r13DEFAULT	422

–	PUSCH-ConfigDedicated-v1130-DEFAULT	422

–	PUSCH-ConfigDedicated-v1250-DEFAULT	422

–	PUSCH-ConfigDedicated--DEFAULT	423

–	PUSCH-EnhancementsConfig-r14-DEFAULT	423

–	RACH-ConfigCommon-DEFAULT	424

–	Rach-ConfigDedicated-DEFAULT	426

–	RadioResourceConfigCommon-DEFAULT	427

–	RadioResourceConfigCommonSCell-r10-DEFAULT	429

–	RadioResourceConfigCommonSIB-DEFAULT	431

–	RadioResourceConfigDedicated-SRB1	432

–	RadioResourceConfigDedicated-SRB2-DRB(n,m)	433

–	RadioResourceConfigDedicated-DRB(n,m)	434

–	RadioResourceConfigDedicated-SRB4	434

–	RadioResourceConfigDedicated-HO-TO-EUTRA(n,m)	435

-	RadioResourceConfigDedicated-AM-DRB-ADD(bid)	436

-	RadioResourceConfigDedicated-UM-DRB-ADD(bid)	436

-	RadioResourceConfigDedicated- DRB-REL(bid)	437

–	RadioResourceConfigDedicated-HO	437

–	RadioResourceConfigDedicatedSCell-r10-DEFAULT	437

–	RadioResourceConfigDedicated-SCell_AddMod	438

–	RadioResourceConfigDedicated-V2X	438

-	RadioResourceConfigDedicated-EN-DC-Split	439

-	RadioResourceConfigDedicated-EN-DC_Split_DRB_Rel(bid)	440

–	RadioResourceConfigDedicated-EN-DC-SRB2-DRB	440

–	RLC-Config-DRB-AM-RECONFIG	441

–	RLC-Config-DRB-UM-RECONFIG	441

–	RLC-Config-SRB-AM-RECONFIG	441

–	SCellToAddMod-r10-DEFAULT	442

–	SCellToRelease-r10-DEFAULT	442

–	SCG-Configuration-r12-DEFAULT	443

–	SchedulingRequest-Config-DEFAULT	446

–	SchedulingRequestConfigSCell-r13-DEFAULT	446

–	SL-CommResourcePoolV2X-r14-DEFAULT	447

–	SL-CommTxPoolSensingConfig-r14-DEFAULT	448

–	SL-InterFreqInfoV2X-r14-DEFAULT	449

–	SlotOrSubslotPDSCH-Config-r15-DEFAULT	449

–	SlotOrSubslotPUSCH-Config-r15-DEFAULT	450

–	SPDCCH-Config-r15-DEFAULT	451

–	SPUCCH-Config-r15-DEFAULT	452

–	SL-PSSCH-TxConfig-r14-DEFAULT	453

–	SL-TxPoolToAddMod-r14-DEFAULT	453

–	SL-TxPoolToReleaseListV2X-r14-DEFAULT	454

–	SL-V2X-ConfigDedicated-r14-DEFAULT	455

–	SL-V2X-InterFreqUE-Config-r14-DEFAULT	457

–	SL-V2X-PreconfigCommPool-r14-DEFAULT	458

–	SL-V2X-PreconfigFreqInfo-r14-DEFAULT	459

–	SoundingRS-UL-ConfigCommon-DEFAULT	460

–	SoundingRS-UL-ConfigDedicated-DEFAULT	460

–	SoundingRS-UL-ConfigDedicatedAperiodic-r10-DEFAULT	461

–	SRB-ToAddModList-RECONFIG	461

–	SRS-TPC-PDCCH-Config-r14-DEFAULT	462

–	TDD-Config-DEFAULT	462

–	TPC-PDCCH-Config-DEFAULT	462

–	TPC-PDCCH-ConfigSCell-r13-DEFAULT	463

–	UplinkPowerControlCommon-DEFAULT	463

–	UplinkPowerControlCommonSCell-r10-DEFAULT	464

–	UplinkPowerControlCommon-v1020-DEFAULT	464

–	UplinkPowerControlCommon-v1530-DEFAULT	464

–	UplinkPowerControlCommonSCell-v1310-DEFAULT	465

–	UplinkPowerControlDedicated-DEFAULT	465

–	UplinkPowerControlDedicated-v1020-DEFAULT	466

–	UplinkPowerControlDedicated-v1130-DEFAULT	466

–	UplinkPowerControlDedicated-v1250-DEFAULT	466

–	UplinkPowerControlDedicated-v1530-DEFAULT	467

–	UplinkPowerControlDedicatedSCell-r10-DEFAULT	467

–	UplinkPowerControlDedicatedSCell-v1310-DEFAULT	467

-	RadioResourceConfigDedicated-DRB-Mod	467

-	RadioResourceConfigDedicated-PCell-PATTERN	468

-	OtherConfig-r9	468

-	WLAN-OffloadConfig-r12	469

–	EIMTA-MainConfig-r12-DEFAULT	470

–	EIMTA-MainConfigServCell-r12-DEFAULT	470

–	LWIP-Configuration-GENERIC	471

–	LWA-Configuration-GENERIC	472

–	LWIP-Configuration-W2-GENERIC	473

–	LWA-Configuration-W2-GENERIC	474

4.6.4	Security control information elements	474

–	SecurityConfigHO-DEFAULT	474

–	SecurityConfigSMC-DEFAULT	475

4.6.5	Mobility control information elements	476

–	MobilityControlInfo-HO	476

4.6.6	Measurement information elements	477

-	MeasConfig-DEFAULT	478

-	MeasGapConfig-GP1	479

-	MeasGapConfig-CE	479

-	MeasGapConfig-GP2	479

-	MeasDS-Config-DEFAULT	480

-	MeasCSI-RS-Config-DEFAULT	480

–	MeasObjectCDMA2000-GENERIC	481

–	ReportConfigToAddModList_DEFAULT	481

–	MeasIdToAddModList_DEFAULT	481

-	MeasObjectEUTRA-GENERIC	482

-	MeasObjectGERAN-GENERIC	483

-	MeasObjectNR-GENERIC	483

-	MeasObjectUTRA-GENERIC	484

–	MeasObjectWLAN-W2-GENERIC	484

-	QuantityConfig-DEFAULT	485

-	ReportConfigEUTRA-A1	486

-	ReportConfigEUTRA-A2	487

-	ReportConfigEUTRA-A3	488

-	ReportConfigEUTRA-A4	489

-	ReportConfigEUTRA-A5	490

-	ReportConfigEUTRA-A6	491

-	ReportConfigEUTRA-PERIODICAL	491

-	ReportConfigInterRAT-B1-GERAN	492

-	ReportConfig-B1-NR-r15	492

-	ReportConfigInterRAT-B1-UTRA	493

-	ReportConfigInterRAT-B2-CDMA2000	494

-	ReportConfigInterRAT-B2-GERAN	495

-	ReportConfigInterRAT-B2-UTRA	496

-	ReportConfigInterRAT-B2-NR	497

-	- ReportConfigInterRAT-SFTD	498

ReportConfigInterRAT-PERIODICAL	498

–	ReportConfigInterRAT-W2-WLAN	499

–	ReportConfigInterRAT-W3-WLAN	499

-	ReportConfigEUTRA-C1	500

-	ReportConfigEUTRA-C2	501

-	ReportConfigEUTRA-PERIODICAL-CSI-RS	502

-	ReportConfigEUTRA-V1	502

-	ReportConfigEUTRA-V2	503

-	ReportConfigEUTRA-H1	503

-	ReportConfigEUTRA-H2	504

4.6.7	Other information elements	504

–	FlightPathInfoReportConfig	504

–	RRC-TransactionIdentifier-DL	504

–	RRC-TransactionIdentifier-UL	505

4.6.8	Channel-bandwidth-dependent parameters	505

4.7	Default NAS message and information element contents	505

4.7.1	Security protected NAS messages	506

4.7.2	Contents of EMM messages	507

-	ATTACH ACCEPT	507

-	ATTACH COMPLETE	510

-	ATTACH REJECT	510

-	ATTACH REQUEST	511

-	AUTHENTICATION FAILURE	512

-	AUTHENTICATION REJECT	513

-	AUTHENTICATION REQUEST	513

-	AUTHENTICATION RESPONSE	514

-	CS SERVICE NOTIFICATION	514

-	CONTROL PLANE SERVICE REQUEST	515

-	DETACH ACCEPT (UE originating detach)	515

-	DETACH ACCEPT (UE terminated detach)	516

-	DETACH REQUEST (UE originating detach)	516

-	DETACH REQUEST (UE terminated detach)	517

-	DOWNLINK NAS TRANSPORT	517

-	EMM INFORMATION	518

-	EMM STATUS	518

-	EXTENDED SERVICE REQUEST	519

-	GUTI REALLOCATION COMMAND	519

-	GUTI REALLOCATION COMPLETE	520

-	IDENTITY REQUEST	520

-	IDENTITY RESPONSE	520

-	SECURITY MODE COMMAND	521

-	SECURITY MODE COMPLETE	522

-	SECURITY MODE REJECT	522

-	SERVICE ACCEPT	522

-	SERVICE REJECT	523

-	SERVICE REQUEST	523

-	TRACKING AREA UPDATE ACCEPT	524

-	TRACKING AREA UPDATE COMPLETE	526

-	TRACKING AREA UPDATE REJECT	527

-	TRACKING AREA UPDATE REQUEST	528

-	UPLINK NAS TRANSPORT	529

4.7.3	Contents of ESM messages	530

-	ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT	530

-	ACTIVATE DEDICATED EPS BEARER CONTEXT REJECT	531

-	ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST	532

-	ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT	533

-	ACTIVATE DEFAULT EPS BEARER CONTEXT REJECT	534

-	ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST	535

-	BEARER RESOURCE ALLOCATION REJECT	540

-	BEARER RESOURCE ALLOCATION REQUEST	540

-	BEARER RESOURCE MODIFICATION REJECT	541

-	BEARER RESOURCE MODIFICATION REQUEST	542

-	DEACTIVATE EPS BEARER CONTEXT ACCEPT	543

-	DEACTIVATE EPS BEARER CONTEXT REQUEST	543

-	ESM DATA TRANSPORT	544

-	ESM DUMMY MESSAGE	544

-	ESM INFORMATION REQUEST	545

-	ESM INFORMATION RESPONSE	546

-	ESM STATUS	547

-	MODIFY EPS BEARER CONTEXT ACCEPT	548

-	MODIFY EPS BEARER CONTEXT REJECT	548

-	MODIFY EPS BEARER CONTEXT REQUEST	549

-	NOTIFICATION	550

-	PDN CONNECTIVITY REJECT	550

-	PDN CONNECTIVITY REQUEST	551

-	PDN DISCONNECT REJECT	552

-	PDN DISCONNECT REQUEST	553

4.7A	Default TC message and information element contents	553

-	ACTIVATE TEST MODE	553

-	ACTIVATE TEST MODE COMPLETE	554

-	CLOSE UE TEST LOOP	555

-	CLOSE UE TEST LOOP COMPLETE	557

-	DEACTIVATE TEST MODE	557

-	DEACTIVATE TEST MODE COMPLETE	557

-	OPEN UE TEST LOOP	558

-	OPEN UE TEST LOOP COMPLETE	558

-	UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST	558

-	UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE	558

-	UE TEST LOOP PROSE PACKET COUNTER REQUEST	559

-	UE TEST LOOP PROSE PACKET COUNTER RESPONSE	559

-	UE TEST LOOP MODE F SCPTM PACKET COUNTER REQUEST	559

-	UE TEST LOOP MODE F SCPTM PACKET COUNTER RESPONSE	560

4.7B	Default UTRA message and information element contents	561

4.7B.1	UTRA RRC messages	561

–	HANDOVER TO UTRAN COMMAND	561

–	HANDOVER FROM UTRAN COMMAND	572

–	MEASUREMENT CONTROL	572

–	MEASUREMENT REPORT	574

–	PHYSICAL CHANNEL RECONFIGURATION	575

–	PHYSICAL CHANNEL RECONFIGURATION COMPLETE	576

–	RRC CONNECTION REQUEST	577

–	SECURITY MODE COMMAND	577

–	SECURITY MODE COMPLETE	578

–	UTRAN MOBILITY INFORMATION	578

–	UTRAN MOBILITY INFORMATION CONFIRM	578

4.7B.2	UTRA NAS messages	578

4.7C	Default DS-MIPv6 message and information element contents	587

4.7C.1	IKEv2 messages	587

-	IKEv2 IKE_SA_INIT Request	587

-	IKE_SA_INIT Response	589

-	IKE_AUTH_Request	590

-	IKE_AUTH Response	592

4.7C.2	Messages used to perform DS-MIPv6 registration and deregistration	595

-	Router Advertisement	595

-	Binding Update	596

-	Binding Acknowledgement	597

-	Binding Revocation Indication	598

-	Binding Revocation Acknowledgement	599

4.7D	Default GERAN message and information element contents	600

4.7D.1	GPRS message	600

–	PS HANDOVER COMMAND	600

4.7E	Default HTTP messages for communication with the ProSe Function	601

-	HTTP Request	601

-	HTTP Response	601

4.7F	Default ProSe messages	602

4.7F.1	ProSe discovery messages	602

-	DISCOVERY_REQUEST	602

-	DISCOVERY_RESPONSE	603

-	MATCH_REPORT	606

-	MATCH_REPORT_ACK	607

-	PC5_DISCOVERY	608

4.7F.2	Messages transmitted over the PC3ch interface	612

-	USAGE_INFORMATION_REPORT_LIST	612

-	USAGE_INFORMATION_REPORT_LIST_RESPONSE	613

4.7F.3	ProSe Direct Communication Messages	614

-	DIRECT_COMMUNICATION_ACCEPT	614

-	DIRECT_COMMUNICATION_KEEPALIVE	614

-	DIRECT_COMMUNICATION_KEEPALIVE_ACK	615

-	DIRECT_COMMUNICATION_RELEASE	615

-	DIRECT_COMMUNICATION_RELEASE_ACCEPT	615

-	DIRECT_COMMUNICATION_REQUEST	616

-	DIRECT_SECURITY_MODE_COMMAND	617

-	DIRECT_SECURITY_MODE_COMPLETE	618

-	KEY_REQUEST	618

-	KEY_RESPONSE	619

-	MIKEY Key Delivery Message	620

-	MIKEY Verification Message	622

4.7G	Default IKEv2 message and information element contents	623

-	IKE_SA_INIT request	623

-	IKE_SA_INIT response	624

-	IKE_AUTH_request	624

-	IKE_AUTH response	625

4.7H	Default TLS message and information element contents	625

-	ClientHello	625

-	ServerHello	625

-	ServerKeyExchange	626

-	ServerHelloDone	626

-	ClientKeyExchange	626

-	ChangeCipherSpec	626

-	Finished	626

4.7I	Default AT Command message and information element	627

-	AT Command +CATM	627

-	AT Command +CCUTLE	627

-	AT Command +CUTCR	627

-	AT Command +CUSPCREQ	628

4.8	Reference radio bearer configurations	628

4.8.1	General	628

4.8.2	SRB and DRB parameters and combinations	628

4.8.2.1	SRB and DRB parameters	628

4.8.2.1.1	SRB configurations	628

4.8.2.1.2	DRB PDCP configurations	629

4.8.2.1.3	DRB RLC configurations	631

4.8.2.1.4	DRB Logical Channel configurations	632

4.8.2.1.5	MAC configurations	633

4.8.2.1.6	Physical Layer configurations	636

4.8.2.1.7	DRB configurations	644

4.8.2.2	SRB and DRB combinations	644

4.8.2.2.1	Combinations on DL-SCH and UL-SCH	644

4.8.3	UTRA reference radio parameters and combinations	644

4.8.4	GERAN reference PDP context parameters	645

4.9	Common test USIM, CSIM and ISIM parameters	645

4.9.1	General	645

4.9.1.1	Definitions	645

4.9.1.2	Definition of the test algorithm for authentication	645

4.9.1.2.1	Authentication and key derivation in the test USIM, CSIM and ISIM and SS	645

4.9.1.2.2	Generation of re-synchronization parameters in the USIM, CSIM and ISIM	646

4.9.1.2.3	Using the authentication test algorithm for UE conformance testing	646

4.9.2	Default parameters for the test USIM, CSIM and ISIM	646

4.9.3	Default settings for the Elementary Files (EFs)	646

4.9.3.1	Modified contents of the USIM Elementary Files and additional USIM Elements files at the DF ProSe level	646

4.9.3.2	Modified contents of the CSIM Elementary Files	653

4.9.3.3	Modified contents of the USIM Elementary Files and additional USIM Elements files at the MCPTT level	664

4.9.3.4	Modified contents of the USIM Elementary Files and additional USIM Elements files at the DFV2X level	665

4.9.3.5	Modified contents of the Elementary Files for eCall over IMS	666

4.10	V2X reference configuration	666

4.10.1	Reference configuration for V2X Sidelink Communication	666

4.10.1.1	V2X Sidelink Communication Preconfiguration for out-of-network coverage operation	666

4.10.2	V2X management data object configuration	667

4.11	GNSS Requirements for V2X and MCS testing	670

4.11.1	General	670

4.11.2	GNSS Scenarios	670

4.12	GNSS Requirements for aerial testing	674

4.12.1	General	674

4.12.2	GNSS Scenarios	674

5	Test environment for RF test	674

5.1	Requirements of test equipment	674

5.2	RF Reference system configurations	674

5.2.1	Common parameters for simulated E-UTRA cells	674

5.2.1.1	Combinations of system information blocks	674

5.2.1.2	Scheduling of system information blocks	675

5.2.1.3	Common contents of system information messages	675

5.2A	Generic RF procedures	677

5.2A.1	UE RF test states	678

5.2A.1A	Registered, Idle Mode, UE Test Mode Activated (State 2A-RF)	679

5.2A.1A.1	Initial conditions	679

5.2A.1A.2	Definition of system information messages	679

5.2A.1A.3	Procedure	680

5.2A.1A.4	Specific message contents	681

5.2A.1AA	Registered, Idle Mode, UE Test Mode Activated in cell supporting BL/CE UE (State 2A-RF-CE)	683

5.2A.1AA.1	Initial conditions	683

5.2A.1AA.2	Definition of system information messages	683

5.2A.1AA.3	Procedure	683

5.2A.1AA.4	Specific message contents	684

5.2A.2	Generic Default Radio Bearer Establishment, UE Test Mode Activated (State 3A-RF)	684

5.2A.2.1	Initial conditions	684

5.2A.2.2	Definition of system information messages	684

5.2A.2.3	Procedure	684

5.2A.2.4	Specific message contents	685

5.2A.2A	DC MCG/SCG Dedicated RB established, UE Test Mode Activate (State 3A-RF-DC1)	685

5.2A.2A.1	Initial conditions	685

5.2A.2A.2	Definition of system information messages	685

5.2A.2A.3	Procedure	685

5.2A.2A.4	Specific message contents	685

5.2A.2AA	Generic Default Radio Bearer Establishment, UE Test Mode Activated in cell supporting BL/CE UE (State 3A-RF-CE)	686

5.2A.2AA.1	Initial conditions	686

5.2A.2AA.2	Definition of system information messages	686

5.2A.2AA.3	Procedure	687

5.2A.2AA.4	Specific message contents	687

5.2A.2B	DC Split Default RB established, UE Test Mode Activate (State 3A-RF-DC2)	688

5.2A.2B.1	Initial conditions	688

5.2A.2B.2	Definition of system information messages	688

5.2A.2B.3	Procedure	688

5.2A.2B.4	Specific message contents	689

5.2A.2C	Generic Default Radio Bearer Establishment, UE Test Mode Activated, V2X Setup (State 3A-RF-V2X)	689

5.2A.2C.1	Initial conditions	689

5.2A.2C.2	Definition of system information messages	689

5.2A.2C.3	Procedure	689

5.2A.2C.4	Specific message contents	689

5.2A.3	Loopback Activation without looped data (State 4A-RF)	689

5.2A.3.1	Initial conditions	689

5.2A.3.2	Definition of system information messages	690

5.2A.3.3	Procedure	690

5.2A.3.4	Specific message contents	690

5.2A.3A	DC MCG/SCG DRBs Loopback Activation without looped data (State 4A-RF-DC1)	690

5.2A.3A.1	Initial conditions	690

5.2A.3A.2	Definition of system information messages	691

5.2A.3A.3	Procedure	691

5.2A.3A.4	Specific message contents	691

5.2A.3AA	Loopback Activation without looped data in cell supporting BL/CE UE (State 4A-RF-CE)	691

5.2A.3AA.1	Initial conditions	691

5.2A.3AA.2	Definition of system information messages	691

5.2A.3AA.3	Procedure	692

5.2A.3AA.4	Specific message contents	692

5.2A.3B	DC Split DRB Loopback Activation without looped data (State 4A-RF-DC2)	692

5.2A.3B.1	Initial conditions	692

5.2A.3B.2	Definition of system information messages	692

5.2A.3B.3	Procedure	692

5.2A.3B.4	Specific message contents	692

5.2A.3C	Loopback Activation without looped data, V2X Setup (State 4A-RF-V2X)	692

5.2A.3C.1	Initial conditions	692

5.2A.3C.2	Definition of system information messages	693

5.2A.3C.3	Procedure	693

5.2A.3C.4	Specific message contents	693

5.2A.4	Procedure to configure SCC	693

5.2 A.41.	Specific message contents	693

5.2A.4.1.1	Exceptions for all CA tests	693

5.2A.4.1.2	Exceptions for UL CA tests	694

5.2A.5	Exceptions for feICIC tests	695

5.2A.5.1	Specific message contents	695

5.2A.5.1.1	Neighbour cell info for all feICIC test cases	695

5.2A.6	Exceptions for NAICS tests	696

5.2A.6.1	NAICS specific RRC Connection reconfiguration procedure	696

5.2A.6.1.1	Procedure	696

5.2A.6.1.1	Specific message contents	696

5.2A.6.2	Specific message contents	696

5.2A.6.2.1	RRCConnectionReconfiguration for setting up and releasing NAICS configuration in NAICS test cases	697

5.2A.7	Procedure to retrieve additional UE Capabilities for Rel-11 and higher UEs that support frequencyBandRetrieval_r11	698

5.2A.7.1	Initial conditions	698

5.2A.7.2	Definition of system information messages	699

5.2A.7.3	Procedure	699

5.2A.7.4	Specific message contents	699

5.2A.8	Exceptions for DL Control channel interference mitigation (Type A and Type B) tests	700

5.2A.8.1	Specific message contents	700

5.2A.8.1.1	Neighbour cell info for all CCIM test cases	700

5.3	Default RRC message and information elements contents	700

5.3.1	Radio resource control information elements	700

5.3.2	Measurement information elements	702

5.4	Default NAS message and information elements contents	702

5.5	Reference radio bearer configurations	702

5.5.1	SRB and DRB parameters	702

5.5.1.1	MAC configurations	702

5.5.1.2	Physical Layer configurations	704

5.5.1.3	SRB and DRB combinations	705

5.5.1.3.1	Combinations on DL-SCH and UL-SCH	705

6	Test environment for Signalling test	706

6.1	Requirements of test equipment	706

6.2	Reference test conditions	706

6.2.1	Physical channel allocations	706

6.2.1.1	Antennas	706

6.2.1.2	Downlink physical channels and physical signals	706

6.2.1.3	Mapping of downlink physical channels and signals to physical resources	707

6.2.1.4	Uplink physical channels and physical signals	707

6.2.1.5	Mapping of uplink physical channels and signals to physical resources	707

6.2.2	Signal levels	707

6.2.2.1	Downlink signal levels	707

6.2.2.2	Measurement accuracy and side conditions	708

6.2.2.3	Uplink signal levels	709

6.2.3	Default test frequencies	710

6.2.3.1	Test frequencies for signalling test	710

6.2.3.2	Test frequencies for CA signalling test	713

6.2.3.3	Test frequencies for ProSe signalling test	725

6.2.3.4	Test frequencies for MFBI frequency band priority adjustment signalling test	726

6.2.3.5	Test frequencies for V2X Communication	726

6.3	Reference system configurations	727

6.3.1	Default parameter specific for simulated cells	727

6.3.1.1	Intra-frequency neighbouring cell list in SIB4 for E-UTRA cells	727

6.3.1.2	Inter-frequency carrier frequency list in SIB5 for E-UTRA cells	727

6.3.1.3	UTRA carrier frequency list in SIB6 for E-UTRA cells	728

6.3.1.4	GERAN carrier frequency group list in SIB7 for E-UTRA cells	729

6.3.1.5	CDMA2000 HRPD carrier frequency list in SIB8 for E-UTRA cells	729

6.3.1.6	CDMA2000 1xRTT carrier frequency list in SIB8 for E-UTRA cells	730

6.3.1.7	E-UTRA carrier frequency list in SIB19 for UTRA cells	730

6.3.1.8	NR carrier frequency list in SIB24 for E-UTRA cells	730

6.3.2	Default configurations for NAS test cases	730

6.3.2.1	Simulated network scenarios for NAS test cases	730

6.3.2.2	Simulated NAS cells	731

6.3.2.3	Broadcast system information	732

6.3.2.3.1	Intra-frequency neighbouring cell list in SIB4 for E-UTRA NAS cells	732

6.3.2.3.2	Inter-frequency carrier frequency list in SIB5 for E-UTRA NAS cells	733

6.3.3	Cell configurations	733

6.3.3.1	Full cell configuration	734

6.3.3.2	Minimum uplink cell configuration	734

6.3.3.3	Broadcast only cell configuration	734

6.3.3.3A	Virtual cell configuration	734

6.3.3.4	Application of different cell configurations	734

6.3.4	SCell configurations	735

6.4	Generic procedures	735

6.4.1	Initial UE states and setup procedures	735

6.4.1.1	Initial UE states and setup procedures	735

6.4.1.2	Dedicated Bearer Establishment (to state 5)	736

6.4.1.2.1	Initial conditions	736

6.4.1.2.2	Definition of system information messages	737

6.4.1.2.3	Procedure	737

6.4.1.2.4	Specific message contents	737

6.4.1.2A	DC MCG/SCG Dedicated Bearer Establishment (to state 5A)	737

6.4.1.2A.1	Initial conditions	737

6.4.1.2A.2	Definition of system information messages	737

6.4.1.2A.3	Procedure	737

6.4.1.2A.4	Specific message contents	738

6.4.1.2B	DC Split Dedicated Bearer Establishment (to state 5B)	738

6.4.1.2B.1	Initial conditions	738

6.4.1.2B.2	Definition of system information messages	738

6.4.1.2B.3	Procedure	738

6.4.1.2B.4	Specific message contents	739

6.4.1.3	Loopback Activation (to state 6)	739

6.4.1.3.1	Initial conditions	739

6.4.1.3.2	Definition of system information messages	739

6.4.1.3.3	Procedure	740

6.4.1.3.4	Specific message contents	740

6.4.1.3A	DC MCG/SCG DRB Loopback Activation (to state 6A)	740

6.4.1.3A.1	Initial conditions	740

6.4.1.3A.2	Definition of system information messages	740

6.4.1.3A.3	Procedure	740

6.4.1.3A.4	Specific message contents	740

6.4.1.3B	DC Split DRB Loopback Activation (to state 6B)	740

6.4.1.3B.1	Initial conditions	740

6.4.1.3B.2	Definition of system information messages	741

6.4.1.3B.3	Procedure	741

6.4.1.3B.4	Specific message contents	741

6.4.2	Test procedures	741

6.4.2.1	Introduction	741

6.4.2.2	Test procedure to check RRC_IDLE state	741

6.4.2.3	Test procedure to check RRC_CONNECTED state	742

6.4.2.4	Test procedure Paging (for NAS testing)	742

6.4.2.5	Test procedure for no response to paging (for NAS testing)	742

6.4.2.6	Test procedure to check that a dedicated EPS bearer context is active (for NAS testing)	743

6.4.2.7	Test procedure to check that UE is camped on a new E-UTRAN cell	743

6.4.2.7A	Test procedure to check that UE is camped on E-UTRAN cell upon mobility from another RAT	744

6.4.2.7B	Test procedure to check that UE is camped on a new E-UTRAN cell / UP CIoT	748

6.4.2.8	Test procedure to check that UE is camped on a new UTRAN cell	748

6.4.2.9	Test procedure to check that UE is camped on a new GERAN cell	749

6.4.2.10	Test procedure to check that UE performs tracking area updating procedure without ISR and security reconfiguration after successful completion of handover from UTRA	750

6.4.3	Reference test procedures for TTCN development	752

6.4.3.1	UE triggered establishment of a dedicated EPS bearer context	753

6.4.3.2	UE triggered establishment of a default EPS bearer context associated with an additional PDN	754

6.4.3.3	UE triggered modification of an EPS bearer context	756

6.4.3.4	UE triggered deletion of an EPS bearer context	757

6.4.3.5	UE triggered CS call	758

6.4.3.6	UE triggered MO SMS over SGs	759

6.4.3.7	CS fallback to UTRAN procedures (LAI of UTRAN cell same as the LAI received in combined Attach procedure in EUTRA cell)	759

6.4.3.7.1	CS fallback to UTRAN with redirection / MT call (PS bearers not established)	760

6.4.3.7.2	CS fallback to UTRAN with redirection / MO call (PS bearers not established)	761

6.4.3.7.3	CS fallback to UTRAN with redirection / MT call (PS bearer established)	762

6.4.3.7.4	CS fallback to UTRAN with redirection / MO call (PS bearer established)	762

6.4.3.7.5	CS fallback to UTRAN with Handover / MT call	763

6.4.3.7.5.1	Specific message contents	764

6.4.3.7.6	CS fallback to UTRAN with Handover / MO call	765

6.4.3.7.6.1	Specific message contents	766

6.4.3.7.7	CS fallback to UTRAN with Handover / emergency call	767

6.4.3.7.7.1	Specific message contents	768

6.4.3.8	CS fallback to GERAN procedures (LAI of GERAN cell same as the LAI received in combined Attach procedure in EUTRA cell)	768

6.4.3.8.1	CS fallback to GERAN with redirection or CCO / MT call (DTM not supported)	769

6.4.3.8.2	CS fallback to GERAN with redirection or CCO / MO call (DTM not supported)	770

6.4.3.8.3	CS fallback to GERAN with PS Handover / MT call (EDTM not supported)	770

6.4.3.8.4	CS fallback to GERAN with PS Handover / MO call (EDTM not supported)	770

6.4.3.8.5	CS fallback to GERAN with PS Handover / MT call (EDTM supported)	770

6.4.3.9	SRVCC Handover to UTRA	770

6.4.3.9.1	Specific message contents	771

6.4.3.10	Offload to WLAN	771

6.4.3.10.1	Specific message contents	772

6.4.3.11	Offload from WLAN	772

6.4.3.11.1	Specific message contents	773

6.4.3.12	Check UE does not offload to WLAN	773

6.4.3.12.1	Specific message contents	773

6.4.3.13	Check UE does not Offload to E-UTRAN	773

6.4.3.14	Procedure for UE initiated detach at non-switch-off	774

6.4.3.13.1	Specific message contents	774

6.4.3A	Test case postambles	774

6.4.3A.1	Introduction	774

6.4.3A.2	Reference end states	774

6.5	Default RRC message and information element contents	777

6.5.1	Measurement information elements	778

-	MeasConfig-DEFAULT	778

-	MeasGapConfig-GP1	778

-	MeasGapConfig-GP2	779

-	MeasGapConfig-CE	779

6.6	Default NAS message and information element contents	779

6.6.1	Reference default EPS bearer contexts	779

6.6.2	Reference dedicated EPS bearer contexts	780

6.6A	Default SMS over SGs message and information element contents	789

6.6A.1	CM-sublayer messages	790

-	CP-ACK	790

-	CP-DATA	790

6.6A.2	Short Message Relay Layer (SM-RL) messages	790

-	RP-ACK RPDU	790

-	RP-DATA RPDU	791

6.6A.3	Short Message Transfer Layer (SM-TL) messages	791

-	SMS-DELIVER	791

-	SMS-SUBMIT	792

6.6B	Reference radio bearer configurations	792

6.6B.1	SRB and DRB parameters and combinations	792

6.6B.1.1	SRB and DRB parameters	792

6.6B.1.1.1	Physical Layer configurations	792

6.7	Timer Tolerances	793

6.8	SideLink reference configuration	793

6.8.1	Reference configuration for Direct Communication	793

6.8.1.1	ProSe Direct Communication Preconfiguration for out-of-network coverage operation	793

6.8.2	Reference configuration for V2X Sidelink Communication	796

6.8.2.1	V2X Sidelink Communication Preconfiguration for out-of-network coverage operation	796

7	Test environment for RRM tests	796

7.1	Requirements of test equipment	796

7.2	RRM Reference system configurations	796

7.2.1	Common parameters for simulated E-UTRA cells	796

7.2.1.1	Combinations of system information blocks	796

7.2.1.2	Scheduling of system information blocks	796

7.2.1.3	Common contents of system information messages	796

7.2.2	Common parameters for simulated GERAN cells	798

7.2.2.1	Mapping of GERAN cells	798

7.2A	Generic RRM procedures	799

7.2A.1	UE RRM test states	799

7.2A.2	UE Registration, UE Test Mode Activated (State 2A-RF)	799

7.2A.2A	UE Registration, UE Test Mode Activated in cell supporting BL/CE UE (State 2A-RF-CE)	799

7.2A.3	Generic Default Radio Bearer Establishment, UE Test Mode Activated (State 3A-RF)	800

7.2A.3A	DC MCG/SCG Dedicated RB established, UE Test Mode Activated (State 3A-RF-DC1)	800

7.2A.3AA	Generic Default Radio Bearer Establishment, UE Test Mode Activated in cell supporting BL/CE UE (State 3A-RF-CE)	800

7.2A.3B	DC Split Default RB established, UE Test Mode Activated (State 3A-RF-DC2)	800

7.2A.3C	Generic Default Radio Bearer Establishment, UE Test Mode Activated, V2X Setup (State 3A-RF-V2X)	800

7.2A.4	Generic Default Radio Bearer Establishment, UE Test Mode Activated, pre-registration on HRPD (State 3B-RF)	800

7.2A.4.1	Initial conditions	800

7.2A.4.2	Definition of system information messages	800

7.2A.4.3	Procedure	801

7.2A.4.4	Specific message contents	801

7.2A.5	Procedure to configure SCC	801

7.2A.6	Exceptions for feICIC tests	801

7.2B	Other generic RRM procedures	801

7.2B.1	Tracking area updating procedure	801

7.3	Default RRC message and information elements contents	803

7.3.1	Contents of RRC messages	803

7.3.2	Radio resource control information elements	803

7.3.3	Measurement information elements	804

7.3A	Default UTRA message and information element contents	805

7.3A.1	UTRA RRC messages	805

7.4	Default NAS message and information elements contents	805

7.5	Reference radio bearer configurations	805

7.5.1	SRB and DRB parameters	805

7.5.1.1	MAC configurations	805

8	NB-IoT test environment	807

8.1	NB-IoT Common test environment	807

8.1.1	NB-IoT Environmental conditions	807

8.1.2	NB-IoT Common requirements of test equipment	807

8.1.3	NB-IoT Reference test conditions	807

8.1.3.1	NB-IoT Test frequencies	807

8.1.3.1.1	NB-IoT FDD Mode Test frequencies	808

8.1.3.1.2	NB-IoT TDD Mode Test frequencies	826

8.1.3.2	NB-IoT Radio conditions	827

8.1.3.2.1	NB-IoT Normal propagation condition	827

8.1.3.3	NB-IoT Physical channel allocations	827

8.1.3.3.1	NB-IoT Antennas	827

8.1.3.3.2	NB-IoT Downlink physical channels and physical signals	827

8.1.3.3.3	NB-IoT Mapping of downlink physical channels and signals to physical resources	828

8.1.3.3.4	NB-IoT Uplink physical channels and physical signals	830

8.1.3.3.5	NB-IoT Mapping of uplink physical channels and signals to physical resources	830

8.1.3.4	NB-IoT Signal levels	830

8.1.3.4.1	NB-IoT Downlink signal levels	830

8.1.3.4.2	NB-IoT Uplink signal levels	830

8.1.3.5	NB-IoT Standard test signals	831

8.1.3.5.1	NB-IoT Downlink test signals	831

8.1.3.5.2	NB-IoT Uplink test signals	831

8.1.3.6	NB-IoT Physical layer parameters	831

8.1.3.6.1	NB-IoT Downlink physical layer parameters	831

8.1.4	NB-IoT Reference system configurations	834

8.1.4.1	NB-IoT Simulated network scenarios	834

8.1.4.1.1	NB-IoT Single cell network scenarios	834

8.1.4.1.2	NB-IoT single mode multi cell network scenarios	834

8.1.4.2	NB-IoT Simulated cells	835

8.1.4.3	NB-IoT Common parameters for simulated cells	838

8.1.4.3.1	NB-IoT Common configurations of system information blocks	838

8.1.4.3.1.1	NB-IoT Combinations of system information blocks	838

8.1.4.3.1.2	NB-IoT Scheduling of system information blocks	839

8.1.4.3.2	NB-IoT Common contents of system information messages	840

-	MasterInformationBlock-NB	840

-	MasterInformationBlock-TDD-NB	842

-	SystemInformation-NB	844

-	SystemInformationBlockType1-NB	845

8.1.4.3.3	NB-IoT Common contents of system information blocks	847

-	SystemInformationBlockType2-NB	847

-	SystemInformationBlockType3-NB	848

-	SystemInformationBlockType4-NB	848

-	SystemInformationBlockType5-NB	849

-	SystemInformationBlockType14-NB	850

-	SystemInformationBlockType15-NB	851

-	SystemInformationBlockType16-NB	851

-	SystemInformationBlockType20-NB	851

-	SystemInformationBlockType22-NB	852

8.1.5	NB-IoT Generic procedures	852

8.1.5.0	General	852

8.1.5.1	NB-IoT UE test states	853

8.1.5.2	NB-IoT UE Attach, Connected mode (State 2-NB)	854

8.1.5.2.0	General	854

8.1.5.2.1	Initial conditions	855

8.1.5.2.2	Definition of system information messages	855

8.1.5.2.3	Procedure	856

8.1.5.2.4	Specific message contents	858

8.1.5.2A	NB-IoT UE Attach, Connected mode, UE Test Mode Activated (State 2A-NB)	858

8.1.5.2A.0	General	858

8.1.5.2A.1	Initial conditions	858

8.1.5.2A.2	Definition of system information messages	859

8.1.5.2A.3	Procedure	859

8.1.5.2A.4	Specific message contents	859

8.1.5.2B	NB-IoT UE Attach, Connected Mode, UE Test Loopback Activated (State 2B-NB)	860

8.1.5.2B.1	Initial conditions	860

8.1.5.2B.2	Definition of system information messages	860

8.1.5.2B.3	Procedure	860

8.1.5.2B.4	Specific message contents	860

8.1.5.3	NB-IoT UE Registered, Idle Mode (State 3-NB)	861

8.1.5.3.1	Initial conditions	861

8.1.5.3.2	Definition of system information messages	862

8.1.5.3.3	Procedure	862

8.1.5.3.4	Specific message contents	862

8.1.5.3A	NB-IoT UE Registered, Idle Mode, UE Test Mode Activated (State 3A-NB)	862

8.1.5.3A.1	Initial conditions	862

8.1.5.3A.2	Definition of system information messages	862

8.1.5.3A.3	Procedure	862

8.1.5.3A.4	Specific message contents	862

8.1.5.4	Void	863

8.1.5A	Other generic procedures	863

8.1.5A.1	Procedure for IP address allocation in the CP CIoT	863

8.1.5A.2	Test procedure to check UE response to Paging for Control Plane CIoT MT access	863

8.1.5A.2.1	Initial conditions	863

8.1.5A.2.2	Definition of system information messages	863

8.1.5A.2.3	Procedure	864

8.1.5A.2.4	Specific message contents	865

8.1.5A.3	Test procedure to check UE initiation of Control Plane CIoT MO user data transfer non-SMS transport	868

8.1.5A.3.1	Initial conditions	868

8.1.5A.3.2	Definition of system information messages	868

8.1.5A.3.3	Procedure	869

8.1.5A.3.4	Specific message contents	869

8.1.5A.3A	Test procedure to check UE initiation of Control Plane CIoT MO user data transfer SMS transport	873

8.1.5A.3A.1	Initial conditions	873

8.1.5A.3A.2	Definition of system information messages	873

8.1.5A.3A.3	Procedure	874

8.1.5A.3A.4	Specific message contents	874

8.1.5A.4	Test procedure to check release of PDN connectivity before leaving RRC-CONNECTED for attach without PDN	878

8.1.5A.4.1	Initial conditions	878

8.1.5A.4.2	Definition of system information messages	878

8.1.5A.4.3	Procedure	879

8.1.5A.4.4	Specific message contents	879

8.1.5A.5	Test procedure to check that NB-IoT UE is camped on a new NB-IOT cell	879

8.1.5A.6	Test procedure to check that NB-IoT UE resume RRC connection on a new NB-IOT cell	880

8.1.5A.7	881

8.1.5A.8	Test procedure to check RRC_CONNECTED state for NB-IoT	881

8.1.5A.9	Test Procedure to establish radio bearers in User Plane	881

8.1.5A.9.1	Initial conditions	881

8.1.5A.9.2	Definition of system information messages	881

8.1.5A.9.3	Procedure	882

8.1.5A.9.4	Specific message contents	882

8.1.6	NB-IoT Default RRC message and information elements contents	882

8.1.6.1	NB-IoT Contents of RRC messages	883

–	DLInformationTransfer-NB	883

–	Paging-NB	883

–	RRCConnectionReconfiguration-NB	884

–	RRCConnectionReconfigurationComplete-NB	884

–	RRCConnectionReestablishment-NB	885

–	RRCConnectionReestablishmentComplete-NB	885

–	RRCConnectionReestablishmentRequest-NB	886

–	RRCConnectionReject-NB	886

–	RRCConnectionRelease-NB	887

–	RRCConnectionRequest-NB	887

–	RRCConnectionResume-NB	888

–	RRCConnectionResumeComplete-NB	888

–	RRCConnectionResumeRequest-NB	889

–	RRCConnectionSetup-NB	889

–	RRCConnectionSetupComplete-NB	890

–	UECapabilityEnquiry-NB	891

–	UECapabilityInformation-NB	891

–	ULInformationTransfer-NB	892

8.1.6.2	NB-IoT System information blocks	892

8.1.6.3	NB-IoT Radio resource control information elements	892

–	BCCH-Config-NB-DEFAULT	892

–	DL-CarrierConfigCommon-NB-DEFAULT	892

–	PCCH-Config-NB-DEFAULT	893

–	NPDCCH-ConfigDedicated-NB-DEFAULT	893

–	NPDSCH-ConfigCommon-NB-DEFAULT	893

–	NPRACH-ConfigSIB-NB-DEFAULT	894

–	NPUSCH-ConfigCommon-NB-DEFAULT	894

–	NPUSCH-ConfigDedicated-NB-DEFAULT	894

–	RACH-ConfigCommon-NB-DEFAULT	895

–	RadioResourceConfigCommonSIB-NB-DEFAULT	895

–	RadioResourceConfigDedicated-NB-SRB	896

–	RadioResourceConfigDedicated-NB-DRB(n)	896

-	RadioResourceConfigDedicated-NB-DRB-ADD(bid)	897

-	RadioResourceConfigDedicated-NB-DRB-REL(bid)	897

–	RLC-Config-NB-SRB-RECONFIG	897

–	SRB-ToAddModList-NB-RECONFIG	898

–	UplinkPowerControlCommon-NB-DEFAULT	898

–	UplinkPowerControlDedicated-NB-DEFAULT	898

-	RadioResourceConfigDedicated-NB-DRB-Mod	898

-	NPRACH-ConfigSIB-NB-v1530-DEFAULT	899

-	NPRACH-ConfigSIB-NB-v1550-DEFAULT	899

8.1.6.4	NB-IoT Security control information elements	899

8.1.6.5	NB-IoT Other information elements	900

–	RRC-TransactionIdentifier-DL	900

–	RRC-TransactionIdentifier-UL	900

8.1.7	NB-IoT Default NAS message and information element contents	900

8.1.7A	NB-IoT Default TC message and information element contents	900

8.1.8	NB-IoT Reference radio bearer configurations	901

8.1.8.1	General	901

8.1.8.2	NB-IoT SRB and DRB parameters and combinations	901

8.1.8.2.1	NB-IoT SRB and DRB parameters	901

8.1.9	NB-IoT Common test USIM parameters	903

8.1.9.1	General	903

8.2	NB-IoT Test environment for RF test	903

8.2.1	NB-IoT Requirements of test equipment	903

8.2.2	NB-IoT RF Reference system configurations	904

8.2.2.1	NB-IoT Common parameters for simulated E-UTRA cells	904

8.2.2.1.1	 NB-IoT Combinations of system information blocks	904

8.2.2.1.2	NB-IoT Scheduling of system information blocks	904

8.2.2.1.3	NB-IoT Common contents of system information messages	904

8.2.2A	NB-IoT Generic RF procedures	904

8.2.3	NB-IoT Default RRC message and information elements contents	904

8.2.3.1	NB-IoT Radio resource control information elements	904

8.2.4	NB-IoT Default NAS message and information elements contents	905

8.2.5	NB-IoT Reference radio bearer configurations	905

8.2.5.1	NB-IoT SRB and DRB parameters	905

8.3	NB-IoT Test environment for Signalling test	905

8.3.1	NB-IoT Requirements of test equipment	905

8.3.2	NB-IoT Reference test conditions	905

8.3.2.1	NB-IoT Physical channel allocations	905

8.3.2.2	NB-IoT Signal levels	906

8.3.2.2.1	NB-IoT Downlink signal levels	906

8.3.2.3	NB-IoT Default test frequencies	906

8.3.2.3.1	NB-IoT Test frequencies for signalling test	906

8.3.3	NB-IoT Reference system configurations	907

8.3.3.1	NB-IoT Default parameters specific for simulated cells	908

8.3.3.1.1	Intra-frequency neighbouring cell list in SIB4-NB for NB-IoT cells	908

8.3.3.1.2	Inter-frequency carrier frequency list in SIB5-NB for NB-IoT cells	908

8.3.3.2	NB-IoT Default configurations for NAS test cases	909

8.3.3.3	NB-IoT Cell configurations	910

8.3.4	NB-IoT Generic signalling procedures	910

8.3.4.1	NB-IoT Initial UE states and setup procedures	910

8.3.4.2	NB-IoT Reference procedures and test procedures for TTCN development	910

8.3.4.3	NB-IoT Test case postambles for TTCN development	910

8.3.5	NB-IoT Default RRC message and information element contents	910

8.3.6	NB-IoT Default NAS message and information element contents	910

8.3.7	NB-IoT Timer tolerances	910

8.4	NB-IOT Test environment for RRM tests	910

8.4.1	NB-IoT Requirements of test equipment	910

8.4.2	NB-IoT RRM Reference system configurations	911

8.4.2.1	NB-IoT Common parameters for simulated NB-IoT cells	911

8.4.2.1.1	NB-IoT Combinations of system information blocks	911

8.4.2.1.2	NB-IoT Scheduling of system information blocks	911

8.4.2.1.3	NB-IoT Common contents of system information messages	911

8.4.2A	NB-IoT Generic RRM procedures	911

8.4.3	NB-IoT Default RRC message and information elements contents	911

8.4.3.1	NB-IoT Radio resource control information elements	911

8.4.4	NB-IoT Default NAS message and information elements contents	911

8.4.5	NB-IoT Reference radio bearer configurations	912

8.4.5.1	NB-IoT SRB and DRB parameters	912

Annex A (informative):	Connection Diagrams	908

Annex B (informative):	Change history	1048

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The definition of the Conformance Tests for UE in E-UTRAN will be a complex task as the complete test suite covers RF, EMC and Protocol aspects of the UE.

Each test requires a Test Environment to be defined in which the UE has to operate to defined standards, constraints and performance. The overall task can be simplified if there are a number of well defined and agreed Common Test Environments where every one can be used for a number of tests. Hence the present document defines testing conditions that are common to several tests avoiding the need to duplicate the same information for every single test.

The present document defines default values for a variety of common areas. Where values are not specified in test cases, the defaults in the present document will apply. If specified, the test case values will take precedence.

# 1	Scope

The present document contains definitions of reference conditions and test signals, default parameters, reference radio bearer configurations used in radio bearer interoperability testing, common radio bearer configurations for other test purposes, common requirements for test equipment and generic set-up procedures for use in conformance tests for the 3rd Generation E-UTRAN User Equipment (UE).

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document unless the context in which the reference is made suggests a different Release is relevant (information on the applicable release in a particular context can be found in e.g. test case title, description or applicability, message description or content).

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.003: "Numbering, addressing and identification".

[3]	3GPP TS 23.122: "Non-Access-Stratum functions related to Mobile Station (MS) in idle mode".

[4]	3GPP TS 24.008: "Mobile radio interface Layer 3 specification; Core network protocols; Stage 3".

[5]	3GPP TS 34.108: "Common Test Environments for User Equipment (UE); Conformance testing".

[6]	3GPP TS 34.109: "Terminal logical test interface; Special conformance testing functions".

[7]	3GPP TS 34.123-1: "User Equipment (UE) conformance specification; Part 1: Protocol conformance specification".

[8]	3GPP TS 34.123-2: "User Equipment (UE) conformance specification; Part 2: Implementation conformance statement (ICS) specification".

[9]	3GPP TS 34.123-3: "User Equipment (UE) conformance specification; Part 3: Abstract test suites (ATSs)".

[10]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[11]	3GPP TS 36.302: "Evolved Universal Terrestrial Radio Access (E-UTRA); Services provided by the physical layer".

[12]	3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode".

[13]	3GPP TS 36.306: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio access capabilities".

[14]	3GPP TS 36.321: "Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification".

[15]	3GPP TS 36.322: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Link Control (RLC) protocol specification".

[16]	3GPP TS 36.323: "Evolved Universal Terrestrial Radio Access (E-UTRA); Packet Data Convergence Protocol (PDCP) specification".

[17]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol Specification".

[18]	3GPP TS 36.523-1: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); User Equipment (UE) conformance specification; Part 1: Protocol conformance specification".

[19]	3GPP TS 36.523-2: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); User Equipment (UE) conformance specification; Part 2: Implementation Conformance Statement (ICS) proforma specification".

[20]	3GPP TS 36.523-3: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); User Equipment (UE) conformance specification; Part 3: Abstract Test Suites (ATS)".

[21]	3GPP TS 36.521-1: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: conformance testing".

[22]	3GPP TS 36.521-2: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Implementation Conformance Statement (ICS)".

[23]	3GPP TR 24.801: "3GPP System Architecture Evolution; CT WG1 aspects".

[24]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Access Network (E-UTRAN) access".

[25]	3GPP TS 51.010-1: "Mobile Station (MS) conformance specification; Part 1: Conformance specification ".

[26]	ISO/IEC 9646 (all parts): "Information technology - Open Systems Interconnection - Conformance testing methodology and framework".

[27]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[28]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[29]	3GPP TS 36.213: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures".

[30]	3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception".

[31]	3GPP TS 33.401: "3GPP System Architecture Evolution (SAE); Security architecture".

[32]	3GPP TS 31.101: "UICC-terminal interface; Physical and logical characteristics".

[33]	3GPP TS 31.102: "Characteristics of the Universal Subscriber Identity Module (USIM) application".

[34]	3GPP TS 36.521-3: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Radio Resource Management conformance testing".

[35]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation".

[36]	3GPP2 TSG-C C.S0038-B v1.0: "Signalling Conformance Specification for High Rate Packet Data Air Interface".

[37]	3GPP2 TSG-C C.S0043-A v1.0: "Signalling Conformance Test Specification for cdma2000 Spread Spectrum Systems".

[38]	3GPP TS 36.509: "Evolved Universal Terrestrial Radio Access (E-UTRA); Special conformance testing functions for User Equipment (UE)"

[39]	3GPP TS 36.133: "Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management"

[40]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3"

[41]	3GPP TS 23.402: “Architecture enhancements for non-3GPP accesses”

[42]	3GPP2 X.S0057-0-B v1.0 “E-UTRAN - eHRPD Connectivity and Interworking: Core Network Aspects”

[43]	3GPP TS 34.229-1: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 1: Protocol conformance specification ".

[44]	3GPP2 C.S0024-B v3.0: "cdma2000 High Rate Packet Data Air Interface Specification".

[45]	3GPP TS 31.103: "Characteristics of the IP Multimedia Services Identity Module (ISIM) application".

[46]	3GPP TS 34.229-1: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 1: Protocol conformance specification".

[47]	3GPP TS 34.229-3: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 3: Abstract Test Suite (ATS)".

[48]	3GPP2 C.S0065-B v2.0: "cdma2000 Application on UICC for Spread Spectrum Systems".

[49]	3GPP2 C.S0005-F v1.0: "Upper Layer (Layer 3) Signalling Standard for cdma2000 Spread Spectrum Systems – Release A, Addendum 2".

[50]	3GPP TS 34.121-1: "User Equipment (UE) conformance specification; Radio transmission and reception (FDD); Part 1: Conformance specification ".

[51]	3GPP TS 34.122: "Terminal conformance specification; Radio transmission and reception (TDD)".

[52]	GSMA PRD IR.92: "IMS Profile for Voice and SMS".

[53]	3GPP TS 24.302: "Access to the 3GPP Evolved Packet Core (EPC)
via non-3GPP access networks; Stage 3".

[54]	Void

[55]	3GPP TS 33.402: "3GPP System Architecture Evolution (SAE);Security aspects of non-3GPP accesses".

[56]	GSMA PRD IR.88: "LTE and EPC Roaming Guidelines".

[57]	IETF RFC 5996: "Internet Key Exchange Protocol Version 2 (IKEv2)".

[58]	3GPP TS 24.334: "Proximity-services (ProSe) User Equipment (UE) to ProSe function protocol aspects; Stage 3".

[59]	3GPP TS 23.303: "Proximity-based services (ProSe); Stage 2"

[60]	3GPP TS 33.303: "Proximity-based Services (ProSe); Security aspects"

[61]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)"

[62]	3GPP TS 36.212: "Multiplexing and channel coding"

[63]	IETF RFC 4346: "The Transport Layer Security (TLS) Protocol Version 1.1".

[64]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[65]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS);Multimedia Telephony;Media handling and interaction".

[66]	3GPP TS 24.483: "Mission Critical Push To Talk (MCPTT) Management Object (MO)".

[67]	3GPP TS 38.508-1: "5GS; Common test environments for User Equipment (UE) conformance testing"

[68]	3GPP TS 37.571-1: "User Equipment (UE) conformance specification for UE positioning; Part 1: Terminal conformance"

[69]	3GPP TS 37.571-5: "User Equipment (UE) conformance specification for UE positioning; Part 5: Test scenarios and assistance data"

[70]	STANAG 4294: "NATO STANAG 4294. Navstar Global Positioning System (GPS) System Characteristics"

[71]	3GPP TS 38.523-1: "5GS; User Equipment (UE) conformance specification; Part 1: Protocol Conformance Specification"

[72]	3GPP TS 38.133: "New Radio (NR); Requirements for support of radio resource management".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] apply, unless specified below:

B: a value followed by "B" is a binary value.

H: a value followed by "H" is a hexadecimal value.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

NDL	Downlink EARFCN

NRB	Transmission bandwidth configuration, expressed in units of resource blocks

NRB_agg 	Aggregated Transmission Bandwidth Configuration. The number of the aggregated RBs within the fully allocated Aggregated Channel bandwidth.

NUL	Uplink EARFCN

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations specified in TR 21.905 [1] apply, with any additional abbreviations specified below:

1xRTT	1x Radio Transmission Technology

BR	Bandwidth Reduced

BR-BCCH	Bandwidth Reduced Broadcast Control Channel

BL	Bandwidth reduced Low complexity

CA	Carrier Aggregation

CC	Component Carrier

CE	Coverage Enhancement

DRB	(user) Data Radio Bearer

EARFCN 	E-UTRA Absolute Radio Frequency Channel Number

ECM	EPS Connection Management

eIMTA	Enhanced Interference Management and Traffic Adaptation

EMM	EPS Mobility Management

ENB	Evolved Node B

EPRE	Energy Per Resource Element

ESM	EPS Session Management

HRPD	High Rate Packet Data

MAC	Media Access Control

MBS	Metropolitan Beacon System

MCS	Mission Critical Services

MTC	Machine Type Communication

OFDM	Orthogonal Frequency Division Multiplexing

RBs	Resource Blocks

ROHC 	Robust Header Compression

SS	System Simulator

TH	Temperature High

TL	Temperature Low

TP	Transmission Point

V2X	Vehicle-to-Everything

VH	Higher extreme Voltage

VL	Lower extreme Voltage

xCH_RA	xCH-to-RS EPRE ratio for the channel xCH in all transmitted OFDM symbols not containing RS

xCH_RB	xCH-to-RS EPRE ratio for the channel xCH in all transmitted OFDM symbols containing RS

# 4	Common test environment

## 4.1	Environmental conditions

The requirements in this clause apply to all types of UE(s).

### 4.1.1	Temperature

The UE shall fulfil all the requirements in the full temperature range of:

Table 4.1.1-1: Temperature Test Environment

| +15C to +35C | For normal conditions (with relative humidity of 25% to 75 %) |
| --- | --- |
| -10C to +55C | For extreme conditions (see IEC publications 68-2-1 and 68-2-2) |

Outside this temperature range the UE, if powered on, shall not make ineffective use of the radio frequency spectrum. In no case shall the UE exceed the transmitted levels as defined in TS 36.101 [27] clause 6.2 for extreme operation.

The normative reference for this requirement is TS 36.101 [27] Annex E.2	.

Some tests are performed also in extreme temperature conditions. These test conditions are denoted as TL (temperature low, -10C) and TH (temperature high, +55C).

### 4.1.2	Voltage

The UE shall fulfil all the requirements in the full voltage range, i.e. the voltage range between the extreme voltages.

The manufacturer shall declare the lower and higher extreme voltages and the approximate shutdown voltage. For the equipment that can be operated from one or more of the power sources listed below, the lower extreme voltage shall not be higher, and the higher extreme voltage shall not be lower than that specified below.

Table 4.1.2-1: Voltage Test Environment

| Power source | Lower extreme voltage | Higher extreme voltage | Normal conditions voltage |
| --- | --- | --- | --- |
| AC mains | 0.9 * nominal | 1.1 * nominal | nominal |
| Regulated lead acid battery | 0.9 * nominal | 1.3 * nominal | 1.1 * nominal |
| Non regulated batteries: Leclanché Lithium Mercury/nickel & cadmium | 0.85 * nominal 0.95 * nominal 0.90 * nominal | nominal 1.1 * nominal | nominal 1.1 * nominal nominal |

Outside this voltage range the UE if powered on, shall not make ineffective use of the radio frequency spectrum. In no case shall the UE exceed the transmitted levels as defined in TS 36.101 [27] for extreme operation. In particular, the UE shall inhibit all RF transmissions when the power supply voltage is below the manufacturer declared shutdown voltage.

The normative reference for this requirement is TS 36.101 [27] Annex E.2.

Some tests are performed also in extreme voltage conditions. These test conditions are denoted as VL (lower extreme voltage) and VH (higher extreme voltage).

## 4.2	Common requirements of test equipment

Mobile conformance testing can be categorized into 3 distinct areas:

-	RF Conformance Testing.

-	EMC Conformance Testing.

-	Signalling Conformance Testing.

The test equipment required for each category of testing may or not be different, depending on the supplier of the test equipment. However, there will be some generic requirements of the test equipment that are essential for all three categories of test, and these are specified in this clause.

In addition, there will be requirements to test operation in multi-system configurations (e.g. EUTRAN plus UTRAN). However, these would not form a common test equipment requirement for the three test areas and are not considered in the present document.

The test equipment shall use the same number of Tx antennas for all parts of the tests, Initial condition and Test procedure, as specified in the minimum requirement.

### 4.2.1	General functional requirements

NOTE:	This clause has been written such that it does not constrain the implementation of different architectures and designs of test equipment.

All test equipment used to perform conformance testing on a UE shall provide a platform suitable for testing UE's that are either:

a)	FDD Mode; or

b)	TDD Mode; or

c)	both FDD/TDD Modes.

All test equipment shall provide (for the mode(s) supported) the following minimum functionality.

-	The capability of emulating a single E-UTRA cell with the appropriate channels to allow the UE to register on the cell.

-	The capability to allow the UE to set up an RRC connection with the system simulator, and to maintain the connection for the duration of the test.

-	The capability (for the specific test):

-	to select and support an appropriate radio bearer for the downlink;

-	to set the appropriate downlink power levels;

-	to set up and support the appropriate radio bearer for the uplink;

-	to set and control the uplink power levels.

### 4.2.2	Minimum functional requirements

#### 4.2.2.1	Supported Cell Configuration

The System Simulator shall provide the capability to simulate a minimum number of cells (of the appropriate E-UTRA Mode) whose number and capabilities are governed by the test cases that need to be performed (test cases are defined in 3GPP TS 36.523-1 [18](Signalling), 3GPP TS 36.521-1 [21] (RF) and 3GPP TS 36.521-3 [34] (RRM). For this purpose test cases can be split into two different categories: Tests that require only one cell and Tests that require several cells.

To perform test cases requiring one cell, the system simulator must provide a cell offering the capabilities to perform all the test cases in this category.

To perform test cases requiring several cells, additional cells must be provided by the system simulator. The additional cells, however, need only provide a minimum set of capabilities so as to support the first cell in carrying out the multicell test cases.

The type and number of channels (especially physical channels) constitute an important set of capabilities for a cell. The following clauses list possible channels that may be supported by the SS. Each channel type, however, and the minimum number of channels needed are only mandatory if specific test cases require them.

The mapping between Logical and Transport channels is as described in 3GPP TS 36.321 [14]. Similarly the mapping between Transport channels and Physical channels is as described in 3GPP TS 36.211, TS 36.302 and TS 36.212. The reference measurement channels (mapping between Transport channels and Physical channels for PDSCH/PDCCH) are defined in 3GPP TS 36.521-1[21] annex A

##### 4.2.2.1.1	Supported Channels

4.2.2.1.1.1	Logical channels

| Logical channel | Minimum number | Comments |
| --- | --- | --- |
| BCCH | 1 |  |
| BR-BCCH | 1 | For testing of BL/CE UE. |
| CCCH | 1 |  |
| DCCH | 2 |  |
| PCCH | 1 |  |
| DTCH | n <FFS> | Depending on SS's support for RB service testing (See clause 12 of 3GPP TS 36.523-1 [18] |

4.2.2.1.1.2	Transport channels

| Transport channel | Minimum number | Comments |
| --- | --- | --- |
| BCH | 1 |  |
| PCH | 1 |  |
| RACH | 1 |  |
| DL-SCH | n <FFS> |  |
| UL-SCH | n <FFS> |  |

4.2.2.1.1.3	Physical channels

| Physical channel | Minimum number | Comments |
| --- | --- | --- |
| PBCH | 1 | Physical Broadcast Channel |
| PCFICH | 1 | The physical control format indicator channel carries information about the number of OFDM symbols used for transmission of PDCCHs in a subframe |
| PDCCH | 1 | The physical downlink control channel carries scheduling assignments and other control information. |
| MPDCCH | 1 | The MTC physical downlink control channel carries scheduling assignments and other control information for BL/CE UE. |
| PDSCH | 1 | Physical Downlink Shared Channel |
| PHICH | [1] | The PHICH carries the hybrid-ARQ ACK/NAK |
| PUCCH | 1 | The physical uplink control channel carries uplink control information |
| PUSCH | 1 | Physical Uplink Shared Channel |
| PRACH | 1 | Physical Random Access Channel |

4.2.2.1.1.4	Physical signals

| Physical signal | Minimum number | Comments |
| --- | --- | --- |
| Demodulation reference signal | NA | UL |
| Sounding Reference signal | NA | UL TBD, if applicable |
| Cell-specific Reference Signal | NA | DL |
| UE-specific reference signal | NA | DL |
| Primary synchronisation signal | NA | DL |
| Secondary synchronisation signal | NA | DL |

#### 4.2.2.2	Support of Tcell timing offset

The timing offset in terms of frame start timing between any pair of TDD cells shall be < [3us]. For FDD cells there is no such restriction.

#### 4.2.2.3	Supported Sidelink Configuration

The System Simulator shall provide the capability to simulate a minimum number of UEs whose number and capabilities are governed by the test cases that need to be performed (test cases are defined in 3GPP TS 36.523-1 [18] (Signalling), 3GPP TS 36.521-1 [21] (RF) and 3GPP TS 36.521-3 [34] (RRM). For this purpose test cases can be split into two different categories: Tests that require only one simulated UE and Tests that require several simulated UEs.

To perform test cases requiring one simulated UE, the system simulator shall provide a UE offering the capabilities to perform all the test cases in this category.

The type and number of sidelink channels (especially physical channels) constitute an important set of capabilities for a simulated UE. The following clauses list possible sidelink channels that may be supported by the SS. Each channel type, however, and the minimum number of channels needed are only mandatory if specific test cases require them.

The mapping between Logical and Transport channels is as described in 3GPP TS 36.321 [14]. Similarly the mapping between Transport channels and Physical channels is as described in 3GPP TS 36.211, TS 36.302 and TS 36.212.

##### 4.2.2.3.1	Supported Sidelink Channels

4.2.2.3.1.1	Logical channels

| Logical channel | Minimum number | Comments |
| --- | --- | --- |
| STCH | 1 |  |
| SBCCH | 1 |  |

4.2.2.3.1.2	Transport channels

| Transport channel | Minimum number | Comments |
| --- | --- | --- |
| SL-BCH | 1 |  |
| SL-DCH | 1 |  |
| SL-SCH | 1 |  |

4.2.2.3.1.3	Physical channels

| Physical channel | Minimum number | Comments |
| --- | --- | --- |
| PSBCH | 1 | Physical Sidelink Broadcast CHannel |
| PSCCH | 1 | Physical Sidelink Control CHannel |
| PSDCH | 1 | Physical Sidelink Discovery CHannel |
| PSSCH | 1 | Physical Sidelink Shared CHannel |

4.2.2.3.1.4	Physical signals

| Physical signal | Minimum number | Comments |
| --- | --- | --- |
| Sidelink Synchronization Signals | NA | Sidelink Synchronization |

## 4.3	Reference test conditions

This clause contains the reference test conditions, which apply to all test cases unless otherwise specified.

### 4.3.1	Test frequencies

The test frequencies are based on the E-UTRA frequency bands defined in the core specifications.

The raster spacing is 100 KHz.

E-UTRA/FDD is designed to operate in paired bands of 3GPP TS 36.101 [27]. The reference test frequencies for the RF and Signalling test environment for each of the operating bands are defined in sub clause 4.3.1.1.

E-UTRA/TDD is designed to operate in unpaired bands of 3GPP TS 36.101 [27]. The reference test frequencies for the RF and Signalling test environment for each of the operating bands are defined in sub clause 4.3.1.2.

For non-CA Signalling testing, E-UTRA frequency to be tested is mid range and E-UTRA channel bandwidth to be tested is 5MHz for all operating bands for all test cases as the default configuration unless specific channel bandwidth is specified for the operating band below:

For Band 11, 13, 18 and 20, channel bandwidth to be tested is 10 MHz as the default configuration.

For Band 38, 39, 40, 41 and 42, channel bandwidth to be tested is 20 MHz as the default configuration.

For operating band 46, the test frequencies defined in Clause 4.3.1.2.14 and 4.3.1.2.14A are derived based on the rules defined in TS 36.104 [30], table 5.7.3-1 Note 3.

For CA Signalling testing with two component carriers, E-UTRA frequencies to be tested are according to sub clause 6.2.3.2 and the E-UTRA channel bandwidth combination is according to Table 4.3.1-1.

Table 4.3.1-1: Default E-UTRA channel bandwidth combinations for CA Signalling testing with two component carriers

| CA configuration (Band(s),BW Class,BW Combination Set) | CA Channel Bandwidth combination carrier 1 + carrier 2 [MHz + MHz] | Comment |
| --- | --- | --- |
| Intra-band contiguous and Bandwidth Class C | 20+20 | This channel bandwidth combination is used for all Intra-band contiguous CA configurations using Bandwidth Class C unless specific channel bandwidth combination is specified for a CA configuration or CA Bandwidth Combination Set by specific entries in this table below. |
| Intra-band contiguous and Bandwidth Class B | 5+5 | This channel bandwidth combination is used for all Intra-band contiguous CA configurations using Bandwidth Class B unless specific channel bandwidth combination is specified for a CA configuration or CA Bandwidth Combination Set by specific entries in this table below. |
| Inter-band CA and CA Bandwidth Class Combination A-A | 10 + 10 | This channel bandwidth combination is used for all Inter-band CA configurations using Bandwidth Class Combination A-A unless specific channel bandwidth combination is specified for a CA configuration or CA Bandwidth Combination Set by specific entries in this table below. |
| CA_1A-19A | 15 + 10 | This channel bandwidth combination is used for Inter-band CA configuration CA_1A-19A. |
| CA_1A-21A | 15 + 15 | This channel bandwidth combination is used for Inter-band CA configuration CA_1A-21A. |
| CA_1A-41A | 10 + 20 | This channel bandwidth combination is used for Inter-band CA configuration CA_1A-41A. |
| CA_1A-42A | 15 + 20 | This channel bandwidth combination is used for Inter-band CA configuration CA_1A-42A. |
| CA_3A-19A | 20 + 10 | This channel bandwidth combination is used for Inter-band CA configuration CA_3A-19A. |
| CA_3A-41A | 20 + 20 | This channel bandwidth combination is used for Inter-band CA configuration CA_3A-41A. |
| CA_3A-42A | 20 + 20 | This channel bandwidth combination is used for Inter-band CA configuration CA_3A-42A. |
| CA_19A-21A | 10 + 15 | This channel bandwidth combination is used for Inter-band CA configuration CA_19A-21A. |
| CA_19A-42A | 10 + 20 | This channel bandwidth combination is used for Inter-band CA configuration CA_19A-42A. |
| CA_21A-42A | 15 + 20 | This channel bandwidth combination is used for Inter-band CA configuration CA_21A-42A. |
| CA_25A-26A | 20 + 20 | This channel bandwidth combination is used for Inter-band CA configuration CA_25A-26A. |
| CA_26A-41A | 10 + 20 | This channel bandwidth combination is used for Inter-band CA configuration CA_26A-41A. |
| CA_28A-41A | 10 + 20 | This channel bandwidth combination is used for Inter-band CA configuration CA_28A-41A. |
| CA_28A-42A | 10 + 20 | This channel bandwidth combination is used for Inter-band CA configuration CA_28A-42A. |
| CA_39A-41A | 20+20 | This channel bandwidth combination is used for Inter-band CA configuration CA_39A-41A. |
| CA_41C-41C | 20 + 20 | This channel bandwidth combination is used for Intra-band CA configuration CA_41C-41C. |
| CA_41A-42A | 20 + 20 | This channel bandwidth combination is used for Inter-band CA configuration CA_41A-42A. |
| CA_39C | 20 + 10 | This channel bandwidth combination is used for Intra-band contiguous CA configuration CA_39C. |

For CA Signalling testing with three component carriers, E-UTRA frequencies to be tested are according to sub clause 6.2.3.2 and the E-UTRA channel bandwidth combination is according to Table 4.3.1-2.

Table 4.3.1-2: Default E-UTRA channel bandwidth combinations for CA Signalling testing with three component carriers

| CA configuration (Band(s),BW Class,BW Combination Set) | CA Channel Bandwidth combination carrier 1 + carrier 2 + carrier 3 [MHz + MHz + MHz] | Comment |
| --- | --- | --- |
| Intra-band contiguous and Bandwidth Class D | 20+20+20 | This channel bandwidth combination is used for all Intra-band contiguous CA configurations using Bandwidth Class D unless specific channel bandwidth combination is specified for a CA configuration or CA Bandwidth Combination Set by specific entries in this table below. |
| Inter-band CA and CA Bandwidth Class Combination A-A-A | 10+10+10 | This channel bandwidth combination is used for all Inter-band contiguous CA configurations using Bandwidth Class Combination A-A-A unless specific channel bandwidth combination is specified for a CA configuration or CA Bandwidth Combination Set by specific entries in this table below. |
| Mixed Inter-band and Intra-band contiguous CA and CA Bandwidth Class Combination A-C or C-A | 10+20+20 (A-C) or 20+20+10 (C-A) | This channel bandwidth combination is used for all mixed Inter-band and Intra-band contiguous CA configurations using Bandwidth Class combination A-C or C-A unless specific channel bandwidth combination is specified for a CA configuration or CA Bandwidth Combination Set by specific entries in this table below. |
| Mixed Inter-band and Intra-band contiguous CA and CA Bandwidth Class Combination A-B or B-A | 10+5+5 (A-B) or 5+5+10 (B-A) | This channel bandwidth combination is used for all mixed Inter-band and Intra-band contiguous CA configurations using Bandwidth Class combination A-B or B-A unless specific channel bandwidth combination is specified for a CA configuration or CA Bandwidth Combination Set by specific entries in this table below. |

For RF testing, E-UTRA frequencies to be tested are low range, mid range and high range for all supported operating bands by default. E-UTRA channel bandwidths to be tested are lowest bandwidth, 5MHz bandwidth and highest bandwidth for all supported operating bands by default. Actual test configurations are specified case by case and stated in test case itself as the initial conditions.

The lowest bandwidth, 5MHz bandwidth and highest bandwidth are selected from the combined table which includes nominal and additional channel bandwidth.

In the case 5MHz bandwidth is not supported by the UE, E-UTRA channel bandwidth to be tested is only lowest bandwidth and highest bandwidth.

If channel bandwidth to be tested is equal to the lowest or highest channel bandwidth, then the same channel bandwidth is not required to be tested twice.

In addition to the default channel bandwidths to be tested specified above, for Bands 2, 4, 18, 19, 20 and 25, an industry requirement of testing in 10MHz channel bandwidth is allowed for test cases in chapters 6 and 7 in TS 36.521-1 [21].

Editor’s note: For RF testing, an industry requirement of testing in 10MHz channel bandwidth is requested for Bands 2, 4, 18, 19, 20 and 25 for test cases in chapters 6 and 7 in TS 36.521-1 [21], changing the existing test points to address this is being discussed in RAN5 and will be considered pending technical justification.

For A-GNSS, MBS, WLAN and BLE testing in TS 37.571-1 [68], the E-UTRA frequency and channel bandwidth to be tested follow the same rules as for Signalling testing above.

For intra-band non-contiguous CA (CA_XA-XA) test frequencies have been defined following the basic assumption that CC1 uses the Low Range frequency of the band, while CC2 the High Range frequency. Additional test frequencies not following this rule, are (should be) identified by notes.

For sidelink direct communication and sidelink direct discovery, operating band and bandwidth are specified in section 5.5D and 5.6D in TS 36.101 [27]. Sidelink direct communication and sidelink direct discovery operates in half duplex mode using the uplink frequency of corresponding band.

For V2X sidelink communication, operating band and bandwidth are specified in section 5.5G and 5.6G in TS 36.101 [27]. V2X sidelink communication operates in half duplex mode using the uplink frequency of corresponding band.

#### 4.3.1.1	FDD Mode Test frequencies

##### 4.3.1.1.1	FDD reference test frequencies for operating band 1

Table 4.3.1.1.1-1: Test frequencies for E-UTRA channel bandwidth for operating band 1

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 5 | 18025 | 1922.5 | 25 | 2112.5 |
|  | 10 | 18050 | 1925 | 50 | 2115 |
|  | 15 | 18075 | 1927.5 | 75 | 2117.5 |
|  | 20 | 18100 | 1930 | 100 | 2120 |
| Mid Range | 5/10/15/20 | 18300 | 1950 | 300 | 2140 |
| High Range | 5 | 18575 | 1977.5 | 575 | 2167.5 |
|  | 10 | 18550 | 1975 | 550 | 2165 |
|  | 15 | 18525 | 1972.5 | 525 | 2162.5 |
|  | 20 | 18500 | 1970 | 500 | 2160 |

##### 4.3.1.1.1A	FDD reference test frequencies for CA in operating band 1

Table 4.3.1.1.1A-1: Test frequencies for CA_1C

| Range | CC-Combo / NRB_agg [RB] |  |  | CC1 Note1 |  |  |  |  | CC2 Note1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL | fUL [MHz] | NDL | fDL [MHz] | BW [RB] | NUL | fUL [MHz] | NDL | fDL [MHz] |
| Low | 75+75 | 75 | 18075 | 1927.5 | 75 | 2117.5 | 75 | 18225 | 1942.5 | 225 | 2132.5 |
|  | 100+100 | 100 | 18100 | 1930 | 100 | 2120 | 100 | 18298 | 1949.8 | 298 | 2139.8 |
| Mid | 75+75 | 75 | 18225 | 1942.5 | 225 | 2132.5 | 75 | 18375 | 1957.5 | 375 | 2147.5 |
|  | 100+100 | 100 | 18201 | 1940.1 | 201 | 2130.1 | 100 | 18399 | 1959.9 | 399 | 2149.9 |
| High | 75+75 | 75 | 18375 | 1957.5 | 375 | 2147.5 | 75 | 18525 | 1972.5 | 525 | 2162.5 |
|  | 100+100 | 100 | 18302 | 1950.2 | 302 | 2140.2 | 100 | 18500 | 1970 | 500 | 2160 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1A-2: Test frequencies for CA_1A-1A

| Test Frequency ID | CC-Combo / NRB_agg [RB] |  |  | CC1 Note1 |  |  | Wgap [MHz] |  |  | CC2 Note1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL | fUL [MHz] | NDL | fDL [MHz] |  | BW [RB] | NUL | fUL [MHz] | NDL | fDL [MHz] |
| Max WGap | 25+25 | 25 | 18025 | 1922.5 | 25 | 2112.5 | 50 | 25 | 18575 | 1977.5 | 575 | 2167.5 |
|  | 25+50 | 25 | 18025 | 1922.5 | 25 | 2112.5 | 45 | 50 | 18550 | 1975 | 550 | 2165 |
|  |  | 50 | 18050 | 1925 | 50 | 2115 | 45 | 25 | 18575 | 1977.5 | 575 | 2167.5 |
|  | 25+75 | 25 | 18025 | 1922.5 | 25 | 2112.5 | 40 | 75 | 18525 | 1972.5 | 525 | 2162.5 |
|  |  | 75 | 18075 | 1927.5 | 75 | 2117.5 | 40 | 25 | 18575 | 1977.5 | 575 | 2167.5 |
|  | 25+100 | 25 | 18025 | 1922.5 | 25 | 2112.5 | 35 | 100 | 18500 | 1970 | 500 | 2160 |
|  |  | 100 | 18100 | 1930 | 100 | 2120 | 35 | 25 | 18575 | 1977.5 | 575 | 2167.5 |
|  | 50+50 | 50 | 18050 | 1925 | 50 | 2115 | 40 | 50 | 18550 | 1975 | 550 | 2165 |
|  | 50+75 | 50 | 18050 | 1925 | 50 | 2115 | 35 | 75 | 18525 | 1972.5 | 525 | 2162.5 |
|  |  | 75 | 18075 | 1927.5 | 75 | 2117.5 | 35 | 50 | 18550 | 1975 | 550 | 2165 |
|  | 50+100 | 50 | 18050 | 1925 | 50 | 2115 | 30 | 100 | 18500 | 1970 | 500 | 2160 |
|  |  | 100 | 18100 | 1930 | 100 | 2120 | 30 | 50 | 18550 | 1975 | 550 | 2165 |
|  | 75+75 | 75 | 18075 | 1927.5 | 75 | 2117.5 | 30 | 75 | 18525 | 1972.5 | 525 | 2162.5 |
|  | 75+100 | 75 | 18075 | 1927.5 | 75 | 2117.5 | 25 | 100 | 18500 | 1970 | 500 | 2160 |
|  |  | 100 | 18100 | 1930 | 100 | 2120 | 25 | 75 | 18525 | 1972.5 | 525 | 2162.5 |
|  | 100+100 | 100 | 18100 | 1930 | 100 | 2120 | 20 | 100 | 18500 | 1970 | 500 | 2160 |
| Refsens2 | 25+100 | 25 | 18175 | 1937.5 | 175 | 2127.5 | 20 | 100 | 18500 | 1970 | 500 | 2160 |
|  | 100+100 | 100 | 18250 | 1945 | 250 | 2135 | 5 | 100 | 18500 | 1970 | 500 | 2160 |
| Note 1: Carriers in increasing frequency order. Note 2: Test point derived with regard to REFSENS requirements. |  |  |  |  |  |  |  |  |  |  |  |  |

##### 4.3.1.1.2	FDD reference test frequencies for operating band 2

Table 4.3.1.1.2-1: Test frequencies for E-UTRA channel bandwidth for operating band 2

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 1.4 | 18607 | 1850.7 | 607 | 1930.7 |
|  | 3 | 18615 | 1851.5 | 615 | 1931.5 |
|  | 5 | 18625 | 1852.5 | 625 | 1932.5 |
|  | 10 | 18650 | 1855 | 650 | 1935 |
|  | 15 [1] | 18675 | 1857.5 | 675 | 1937.5 |
|  | 20 [1] | 18700 | 1860 | 700 | 1940 |
| Mid Range | 1.4/3/5/10 15 [1]/20 [1] | 18900 | 1880 | 900 | 1960 |
| High Range | 1.4 | 19193 | 1909.3 | 1193 | 1989.3 |
|  | 3 | 19185 | 1908.5 | 1185 | 1988.5 |
|  | 5 | 19175 | 1907.5 | 1175 | 1987.5 |
|  | 10 | 19150 | 1905 | 1150 | 1985 |
|  | 15 [1] | 19125 | 1902.5 | 1125 | 1982.5 |
|  | 20 [1] | 19100 | 1900 | 1100 | 1980 |
| NOTE 1: Bandwidth for which a relaxation of the specified UE receiver sensitivity requirement (TS 36.101 [27] Clause 7.3) is allowed. |  |  |  |  |  |

##### 4.3.1.1.2A	FDD reference test frequencies for CA in operating band 2

Table 4.3.1.1.2A-1: Test frequencies for CA_2A-2A

| Test Frequency ID | CC-Combo / NRB_agg [RB] |  |  | CC1 Note1 |  |  | Wgap [MHz] |  |  | CC2 Note1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL | fUL [MHz] | NDL | fDL [MHz] |  | BW [RB] | NUL | fUL [MHz] | NDL | fDL [MHz] |
| Max WGap | 25+25 | 25 | 18625 | 1852.5 | 625 | 1932.5 | 50 | 25 | 19175 | 1907.5 | 1175 | 1987.5 |
|  | 25+50 | 25 | 18625 | 1852.5 | 625 | 1932.5 | 45 | 50 | 19150 | 1905 | 1150 | 1985 |
|  |  | 50 | 18650 | 1855 | 650 | 1935 | 45 | 25 | 19175 | 1907.5 | 1175 | 1987.5 |
|  | 25+75 | 25 | 18625 | 1852.5 | 625 | 1932.5 | 40 | 75 | 19125 | 1902.5 | 1125 | 1982.5 |
|  |  | 75 | 18675 | 1857.5 | 675 | 1937.5 | 40 | 25 | 19175 | 1907.5 | 1175 | 1987.5 |
|  | 50+50 | 50 | 18650 | 1855 | 650 | 1935 | 40 | 50 | 19150 | 1905 | 1150 | 1985 |
|  | 25+100 | 25 | 18625 | 1852.5 | 625 | 1932.5 | 35 | 100 | 19100 | 1900 | 1100 | 1980 |
|  |  | 100 | 18700 | 1860 | 700 | 1940 | 35 | 25 | 19175 | 1907.5 | 1175 | 1987.5 |
|  | 50+75 | 50 | 18650 | 1855 | 650 | 1935 | 35 | 75 | 19125 | 1902.5 | 1125 | 1982.5 |
|  |  | 75 | 18675 | 1857.5 | 675 | 1937.5 | 35 | 50 | 19150 | 1905 | 1150 | 1985 |
|  | 50+100 | 50 | 18650 | 1855 | 650 | 1935 | 30 | 100 | 19100 | 1900 | 1100 | 1980 |
|  |  | 100 | 18700 | 1860 | 700 | 1940 | 30 | 50 | 19150 | 1905 | 1150 | 1985 |
|  | 75+75 | 75 | 18675 | 1857.5 | 675 | 1937.5 | 30 | 75 | 19125 | 1902.5 | 1125 | 1982.5 |
|  | 75+100 | 75 | 18675 | 1857.5 | 675 | 1937.5 | 25 | 100 | 19100 | 1900 | 1100 | 1980 |
|  |  | 100 | 18700 | 1860 | 700 | 1940 | 25 | 75 | 19125 | 1902.5 | 1125 | 1982.5 |
|  | 100+100 | 100 | 18700 | 1860 | 700 | 1940 | 20 | 100 | 19100 | 1900 | 1100 | 1980 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.2A-2: Test frequencies for CA_2C

| Range | CC-Combo / NRB_agg [RB] |  |  |  |  | CC1 Note1 |  |  |  |  |  |  |  | CC2 Note1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL |  | fUL [MHz] |  | NDL |  | fDL [MHz] |  | BW [RB] | NUL |  | fUL [MHz] | NDL | fDL [MHz] |  |
| Low | 25+100 | 25 | 18633 |  | 1853.3 |  | 633 |  | 1933.3 |  | 100 | 18750 |  | 1865 | 750 | 1945 |  |
|  |  | 100 | 18700 |  | 1860 |  | 700 |  | 1940 |  | 25 | 18817 |  | 1871.7 | 817 | 1951.7 |  |
|  | 50+75 | 50 | 18653 |  | 1855.3 |  | 653 |  | 1935.3 |  | 75 | 18773 |  | 1867.3 | 773 | 1947.3 |  |
|  |  | 75 | 18675 |  | 1857.5 |  | 675 |  | 1937.5 |  | 50 | 18795 |  | 1869.5 | 795 | 1949.5 |  |
|  | 50+100 | 50 | 18655 |  | 1855.5 |  | 655 |  | 1935.5 |  | 100 | 18799 |  | 1869.9 | 799 | 1949.9 |  |
|  |  | 100 | 18700 |  | 1860 |  | 700 |  | 1940 |  | 50 | 18844 |  | 1874.4 | 844 | 1954.4 |  |
|  | 75+75 | 75 | 18675 |  | 1857.5 |  | 675 |  | 1937.5 |  | 75 | 18825 |  | 1872.5 | 825 | 1952.5 |  |
|  | 75+100 | 75 | 18678 |  | 1857.8 |  | 678 |  | 1937.8 |  | 100 | 18849 |  | 1874.9 | 849 | 1954.9 |  |
|  |  | 100 | 18700 |  | 1860 |  | 700 |  | 1940 |  | 75 | 18871 |  | 1877.1 | 871 | 1957.1 |  |
|  | 100+100 | 100 | 18700 |  | 1860 |  | 700 |  | 1940 |  | 100 | 18898 |  | 1879.8 | 898 | 1959.8 |  |
| Mid | 25+100 | 25 | 18808 |  | 1870.8 |  | 808 |  | 1950.8 |  | 100 | 18925 |  | 1882.5 | 925 | 1962.5 |  |
|  |  | 100 | 18875 |  | 1877.5 |  | 875 |  | 1957.5 |  | 25 | 18992 |  | 1889.2 | 992 | 1969.2 |  |
|  | 50+75 | 50 | 18829 |  | 1872.9 |  | 829 |  | 1952.9 |  | 75 | 18949 |  | 1884.9 | 949 | 1964.9 |  |
|  |  | 75 | 18851 |  | 1875.1 |  | 851 |  | 1955.1 |  | 50 | 18971 |  | 1887.1 | 971 | 1967.1 |  |
|  | 50+100 | 50 | 18806 |  | 1870.6 |  | 806 |  | 1950.6 |  | 100 | 18950 |  | 1885 | 950 | 1965 |  |
|  |  | 100 | 18851 |  | 1875.1 |  | 851 |  | 1955.1 |  | 50 | 18995 |  | 1889.5 | 995 | 1969.5 |  |
|  | 75+75 | 75 | 18825 |  | 1872.5 |  | 825 |  | 1952.5 |  | 75 | 18975 |  | 1887.5 | 975 | 1967.5 |  |
|  | 75+100 | 75 | 18803 |  | 1870.3 |  | 803 |  | 1950.3 |  | 100 | 18974 |  | 1887.4 | 974 | 1967.4 |  |
|  |  | 100 | 18826 |  | 1872.6 |  | 826 |  | 1952.6 |  | 75 | 18997 |  | 1889.7 | 997 | 1969.7 |  |
|  | 100+100 | 100 | 18801 |  | 1870.1 |  | 801 |  | 1950.1 |  | 100 | 18999 |  | 1889.9 | 999 | 1969.9 |  |
| High | 25+100 | 25 | 18983 |  | 1888.3 |  | 983 |  | 1968.3 |  | 100 | 19100 |  | 1900 | 1100 | 1980 |  |
|  |  | 100 | 19050 |  | 1895 |  | 1050 |  | 1975 |  | 25 | 19167 |  | 1906.7 | 1167 | 1986.7 |  |
|  | 50+75 | 50 | 19005 |  | 1890.5 |  | 1005 |  | 1970.5 |  | 75 | 19125 |  | 1902.5 | 1125 | 1982.5 |  |
|  |  | 75 | 19027 |  | 1892.7 |  | 1027 |  | 1972.7 |  | 50 | 19147 |  | 1904.7 | 1147 | 1984.7 |  |
|  | 50+100 | 50 | 18956 |  | 1885.6 |  | 956 |  | 1965.6 |  | 100 | 19100 |  | 1900 | 1100 | 1980 |  |
|  |  | 100 | 19001 |  | 1890.1 |  | 1001 |  | 1970.1 |  | 50 | 19145 |  | 1904.5 | 1145 | 1984.5 |  |
|  | 75+75 | 75 | 18975 |  | 1887.5 |  | 975 |  | 1967.5 |  | 75 | 19125 |  | 1902.5 | 1125 | 1982.5 |  |
|  | 75+100 | 75 | 18929 |  | 1882.9 |  | 929 |  | 1962.9 |  | 100 | 19100 |  | 1900 | 1100 | 1980 |  |
|  |  | 100 | 18951 |  | 1885.1 |  | 951 |  | 1965.1 |  | 75 | 19122 |  | 1902.2 | 1122 | 1982.2 |  |
|  | 100+100 | 100 | 18902 |  | 1880.2 |  | 902 |  | 1960.2 |  | 100 | 19100 |  | 1900 | 1100 | 1980 |  |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### 4.3.1.1.3	FDD reference test frequencies for operating band 3

Table 4.3.1.1.3-1: Test frequencies for E-UTRA channel bandwidth for operating band 3

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 1.4 | 19207 | 1710.7 | 1207 | 1805.7 |
|  | 3 | 19215 | 1711.5 | 1215 | 1806.5 |
|  | 5 | 19225 | 1712.5 | 1225 | 1807.5 |
|  | 10 | 19250 | 1715 | 1250 | 1810 |
|  | 15 [1] | 19275 | 1717.5 | 1275 | 1812.5 |
|  | 20 [1] | 19300 | 1720 | 1300 | 1815 |
| Mid Range | 1.4/3/5/10 15 [1]/20 [1] | 19575 | 1747.5 | 1575 | 1842.5 |
| High Range | 1.4 | 19943 | 1784.3 | 1943 | 1879.3 |
|  | 3 | 19935 | 1783.5 | 1935 | 1878.5 |
|  | 5 | 19925 | 1782.5 | 1925 | 1877.5 |
|  | 10 | 19900 | 1780 | 1900 | 1875 |
|  | 15 [1] | 19875 | 1777.5 | 1875 | 1872.5 |
|  | 20 [1] | 19850 | 1775 | 1850 | 1870 |
| NOTE 1: Bandwidth for which a relaxation of the specified UE receiver sensitivity requirement (TS 36.101 [27] Clause 7.3) is allowed. |  |  |  |  |  |

##### 4.3.1.1.3A	FDD reference test frequencies for CA in operating band 3

Table 4.3.1.1.3A-1: Test frequencies for CA_3C

| Range | CC-Combo / NRB_agg [RB] |  |  |  |  | CC1 Note1 |  |  |  |  |  |  |  | CC2 Note1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL |  | fUL [MHz] |  | NDL |  | fDL [MHz] |  | BW [RB] | NUL |  | fUL [MHz] | NDL | fDL [MHz] |  |
| Low | 25+100 | 25 | 19233 |  | 1713.3 |  | 1233 |  | 1808.3 |  | 100 | 19350 |  | 1725 | 1350 | 1820 |  |
|  |  | 100 | 19300 |  | 1720 |  | 1300 |  | 1815 |  | 25 | 19417 |  | 1731.7 | 1417 | 1826.7 |  |
|  | 50+100 | 50 | 19255 |  | 1715.5 |  | 1255 |  | 1810.5 |  | 100 | 19399 |  | 1729.9 | 1399 | 1824.9 |  |
|  |  | 100 | 19300 |  | 1720 |  | 1300 |  | 1815 |  | 50 | 19444 |  | 1734.4 | 1444 | 1829.4 |  |
|  | 75+100 | 75 | 19278 |  | 1717.8 |  | 1278 |  | 1812.8 |  | 100 | 19449 |  | 1734.9 | 1449 | 1829.9 |  |
|  |  | 100 | 19300 |  | 1720 |  | 1300 |  | 1815 |  | 75 | 19471 |  | 1737.1 | 1471 | 1832.1 |  |
|  | 100+100 | 100 | 19300 |  | 1720 |  | 1300 |  | 1815 |  | 100 | 19498 |  | 1739.8 | 1498 | 1834.8 |  |
| Mid | 25+100 | 25 | 19483 |  | 1738.3 |  | 1483 |  | 1833.3 |  | 100 | 19600 |  | 1750.0 | 1600 | 1845.0 |  |
|  |  | 100 | 19550 |  | 1745 |  | 1550 |  | 1840 |  | 25 | 19667 |  | 1756.7 | 1667 | 1851.7 |  |
|  | 50+100 | 50 | 19481 |  | 1738.1 |  | 1481 |  | 1833.1 |  | 100 | 19625 |  | 1752.5 | 1625 | 1847.5 |  |
|  |  | 100 | 19526 |  | 1742.6 |  | 1526 |  | 1837.6 |  | 50 | 19670 |  | 1757.0 | 1670 | 1852.0 |  |
|  | 75+100 | 75 | 19478 |  | 1737.8 |  | 1478 |  | 1832.8 |  | 100 | 19649 |  | 1754.9 | 1649 | 1849.9 |  |
|  |  | 100 | 19501 |  | 1740.1 |  | 1501 |  | 1835.1 |  | 75 | 19672 |  | 1757.2 | 1672 | 1852.2 |  |
|  | 100+100 | 100 | 19476 |  | 1737.6 |  | 1476 |  | 1832.6 |  | 100 | 19674 |  | 1757.4 | 1674 | 1852.4 |  |
| High | 25+100 | 25 | 19733 |  | 1763.3 |  | 1733 |  | 1858.3 |  | 100 | 19850 |  | 1775 | 1850 | 1870 |  |
|  |  | 100 | 19800 |  | 1770 |  | 1800 |  | 1865 |  | 25 | 19917 |  | 1781.7 | 1917 | 1876.7 |  |
|  | 50+100 | 50 | 19706 |  | 1760.6 |  | 1706 |  | 1855.6 |  | 100 | 19850 |  | 1775 | 1850 | 1870 |  |
|  |  | 100 | 19751 |  | 1765.1 |  | 1751 |  | 1860.1 |  | 50 | 19895 |  | 1779.5 | 1895 | 1874.5 |  |
|  | 75+100 | 75 | 19679 |  | 1757.9 |  | 1679 |  | 1852.9 |  | 100 | 19850 |  | 1775 | 1850 | 1870 |  |
|  |  | 100 | 19701 |  | 1760.1 |  | 1701 |  | 1855.1 |  | 75 | 19872 |  | 1777.2 | 1872 | 1872.2 |  |
|  | 100+100 | 100 | 19652 |  | 1755.2 |  | 1652 |  | 1850.2 |  | 100 | 19850 |  | 1775 | 1850 | 1870 |  |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.3A-2: Test frequencies for CA_3A-3A

| Test Frequency ID | CC-Combo / NRB_agg [RB] |  |  | CC1 Note1 |  |  | Wgap [MHz] |  |  | CC2 Note1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL | fUL [MHz] | NDL | fDL [MHz] |  | BW [RB] | NUL | fUL [MHz] | NDL | fDL [MHz] |
| Max WGap | 15+25 | 15 | 19215 | 1711.5 | 1215 | 1806.5 | 67 | 25 | 19925 | 1782.5 | 1925 | 1877.5 |
|  |  | 25 | 19225 | 1712.5 | 1225 | 1807.5 | 67 | 15 | 19935 | 1783.5 | 1935 | 1878.5 |
|  | 25+25 | 25 | 19225 | 1712.5 | 1225 | 1807.5 | 65 | 25 | 19925 | 1782.5 | 1925 | 1877.5 |
|  | 25+50 | 25 | 19225 | 1712.5 | 1225 | 1807.5 | 60 | 50 | 19900 | 1780 | 1900 | 1875 |
|  |  | 50 | 19250 | 1715 | 1250 | 1810 | 60 | 25 | 19925 | 1782.5 | 1925 | 1877.5 |
|  | 25+75 | 25 | 19225 | 1712.5 | 1225 | 1807.5 | 55 | 75 | 19875 | 1777.5 | 1875 | 1872.5 |
|  |  | 75 | 19275 | 1717.5 | 1275 | 1812.5 | 55 | 25 | 19925 | 1782.5 | 1925 | 1877.5 |
|  | 50+50 | 50 | 19250 | 1715 | 1250 | 1810 | 55 | 50 | 19900 | 1780 | 1900 | 1875 |
|  | 25+100 | 25 | 19225 | 1712.5 | 1225 | 1807.5 | 50 | 100 | 19850 | 1775 | 1850 | 1870 |
|  |  | 100 | 19300 | 1720 | 1300 | 1815 | 50 | 25 | 19925 | 1782.5 | 1925 | 1877.5 |
|  | 50+75 | 50 | 19250 | 1715 | 1250 | 1810 | 50 | 75 | 19875 | 1777.5 | 1875 | 1872.5 |
|  |  | 75 | 19275 | 1717.5 | 1275 | 1812.5 | 50 | 50 | 19900 | 1780 | 1900 | 1875 |
|  | 50+100 | 50 | 19250 | 1715 | 1250 | 1810 | 45 | 100 | 19850 | 1775 | 1850 | 1870 |
|  |  | 100 | 19300 | 1720 | 1300 | 1815 | 45 | 50 | 19900 | 1780 | 1900 | 1875 |
|  | 75+75 | 75 | 19275 | 1717.5 | 1275 | 1812.5 | 45 | 75 | 19875 | 1777.5 | 1875 | 1872.5 |
|  | 75+100 | 75 | 19275 | 1717.5 | 1275 | 1812.5 | 40 | 100 | 19850 | 1775 | 1850 | 1870 |
|  |  | 100 | 19300 | 1720 | 1300 | 1815 | 40 | 75 | 19875 | 1777.5 | 1875 | 1872.5 |
|  | 100+100 | 100 | 19300 | 1720 | 1300 | 1815 | 35 | 100 | 19850 | 1775 | 1850 | 1870 |
| Refsens2 | 25+100 | 25 | 19575 | 1747.5 | 1575 | 1842.5 | 15 | 100 | 19850 | 1775 | 1850 | 1870 |
|  | 50 + 100 | 50 | 19600 | 1750 | 1600 | 1845 | 10 | 100 | 19850 | 1775 | 1850 | 1870 |
| Note 1: Carriers in increasing frequency order. Note 2: Test point derived with regard to REFSENS requirements. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.3A-3: Test frequencies for CA_3B

| Range | CC-Combo / NRB_agg [RB] |  |  |  |  | CC1 Note1 |  |  |  |  |  |  |  | CC2 Note1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL |  | fUL [MHz] |  | NDL |  | fDL [MHz] |  | BW [RB] | NUL |  | fUL [MHz] | NDL | fDL [MHz] |  |
| Low | 15+25 | 15 | 19216 |  | 1711.6 |  | 1216 |  | 1806.6 |  | 25 | 19255 |  | 1715.5 | 1255 | 1810.5 |  |
|  |  | 25 | 19225 |  | 1712.5 |  | 1225 |  | 1807.5 |  | 15 | 19264 |  | 1716.4 | 1264 | 1811.4 |  |
|  | 25+25 | 25 | 19225 |  | 1712.5 |  | 1225 |  | 1807.5 |  | 25 | 19273 |  | 1717.3 | 1273 | 1812.3 |  |
| Mid | 15+25 | 15 | 19551 |  | 1745.1 |  | 1551 |  | 1840.1 |  | 25 | 19590 |  | 1749.0 | 1590 | 1844.0 |  |
|  |  | 25 | 19560 |  | 1746.0 |  | 1560 |  | 1841.0 |  | 15 | 19599 |  | 1749.9 | 1599 | 1844.9 |  |
|  | 25+25 | 25 | 19551 |  | 1745.1 |  | 1551 |  | 1840.1 |  | 25 | 19599 |  | 1749.9 | 1599 | 1844.9 |  |
| High | 15+25 | 15 | 19886 |  | 1778.6 |  | 1886 |  | 1873.6 |  | 25 | 19925 |  | 1782.5 | 1925 | 1877.5 |  |
|  |  | 25 | 19895 |  | 1779.5 |  | 1895 |  | 1874.5 |  | 15 | 19934 |  | 1783.4 | 1934 | 1878.4 |  |
|  | 25+25 | 25 | 19877 |  | 1777.7 |  | 1877 |  | 1872.7 |  | 25 | 19925 |  | 1782.5 | 1925 | 1877.5 |  |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### 4.3.1.1.4	FDD reference test frequencies for operating band 4

Table 4.3.1.1.4-1: Test frequencies for E-UTRA channel bandwidth for operating band 4

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 1.4 | 19957 | 1710.7 | 1957 | 2110.7 |
|  | 3 | 19965 | 1711.5 | 1965 | 2111.5 |
|  | 5 | 19975 | 1712.5 | 1975 | 2112.5 |
|  | 10 | 20000 | 1715 | 2000 | 2115 |
|  | 15 | 20025 | 1717.5 | 2025 | 2117.5 |
|  | 20 | 20050 | 1720 | 2050 | 2120 |
| Mid Range | 1.4/3/5/10/15/20 | 20175 | 1732.5 | 2175 | 2132.5 |
| High Range | 1.4 | 20393 | 1754.3 | 2393 | 2154.3 |
|  | 3 | 20385 | 1753.5 | 2385 | 2153.5 |
|  | 5 | 20375 | 1752.5 | 2375 | 2152.5 |
|  | 10 | 20350 | 1750 | 2350 | 2150 |
|  | 15 | 20325 | 1747.5 | 2325 | 2147.5 |
|  | 20 | 20300 | 1745 | 2300 | 2145 |

##### 4.3.1.1.4A	FDD reference test frequencies for CA in operating band 4

Table 4.3.1.1.4A-1: Test frequencies for CA_4A-4A

| Test Frequency ID | CC-Combo / NRB_agg [RB] |  |  | CC1 Note1 |  |  | Wgap [MHz] |  |  | CC2 Note1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL | fUL [MHz] | NDL | fDL [MHz] |  | BW [RB] | NUL | fUL [MHz] | NDL | fDL [MHz] |
| Low WGap | 25+25 | 25 | 20125 | 1727.5 | 2125 | 2127.5 | 5 | 25 | 20225 | 1737.5 | 2225 | 2137.5 |
|  | 50+50 | 50 | 20100 | 1725 | 2100 | 2125 | 5 | 50 | 20250 | 1740 | 2250 | 2140 |
|  | 50+100 | 50 | 20095 | 1724.5 | 2095 | 2124.5 | 5 | 100 | 20255 | 1740.5 | 2255 | 2140.5 |
|  | 100+100 | 100 | 20050 | 1720 | 2050 | 2120 | 5 | 100 | 20300 | 1745 | 2300 | 2145 |
| Max WGap | 25+25 | 25 | 19975 | 1712.5 | 1975 | 2112.5 | 35 | 25 | 20375 | 1752.5 | 2375 | 2152.5 |
|  | 25+50 | 25 | 19975 | 1712.5 | 1975 | 2112.5 | 30 | 50 | 20350 | 1750 | 2350 | 2150 |
|  |  | 50 | 20000 | 1715 | 2000 | 2115 | 30 | 25 | 20375 | 1752.5 | 2375 | 2152.5 |
|  | 25+75 | 25 | 19975 | 1712.5 | 1975 | 2112.5 | 25 | 75 | 20325 | 1747.5 | 2325 | 2147.5 |
|  |  | 75 | 20025 | 1717.5 | 2025 | 2117.5 | 25 | 25 | 20375 | 1752.5 | 2375 | 2152.5 |
|  | 50+50 | 50 | 20000 | 1715 | 2000 | 2115 | 25 | 50 | 20350 | 1750 | 2350 | 2150 |
|  | 25+100 | 25 | 19975 | 1712.5 | 1975 | 2112.5 | 20 | 100 | 20300 | 1745 | 2300 | 2145 |
|  |  | 100 | 20050 | 1720 | 2050 | 2120 | 20 | 25 | 20375 | 1752.5 | 2375 | 2152.5 |
|  | 50+75 | 50 | 20000 | 1715 | 2000 | 2115 | 20 | 75 | 20325 | 1747.5 | 2325 | 2147.5 |
|  |  | 75 | 20025 | 1717.5 | 2025 | 2117.5 | 20 | 50 | 20350 | 1750 | 2350 | 2150 |
|  | 50+100 | 50 | 20000 | 1715 | 2000 | 2115 | 15 | 100 | 20300 | 1745 | 2300 | 2145 |
|  |  | 100 | 20050 | 1720 | 2050 | 2120 | 15 | 50 | 20350 | 1750 | 2350 | 2150 |
|  | 75+75 | 75 | 20025 | 1717.5 | 2025 | 2117.5 | 15 | 75 | 20325 | 1747.5 | 2325 | 2147.5 |
|  | 75+100 | 75 | 20025 | 1717.5 | 2025 | 2117.5 | 10 | 100 | 20300 | 1745 | 2300 | 2145 |
|  |  | 100 | 20050 | 1720 | 2050 | 2120 | 10 | 75 | 20325 | 1747.5 | 2325 | 2147.5 |
|  | 100+100 | 100 | 20050 | 1720 | 2050 | 2120 | 5 | 100 | 20300 | 1745 | 2300 | 2145 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |  |  |

##### 4.3.1.1.5	FDD reference test frequencies for operating band 5

Table 4.3.1.1.5-1: Test frequencies for E-UTRA channel bandwidth for operating band 5

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 1.4 | 20407 | 824.7 | 2407 | 869.7 |
|  | 3 | 20415 | 825.5 | 2415 | 870.5 |
|  | 5 | 20425 | 826.5 | 2425 | 871.5 |
|  | 10 [1] | 20450 | 829 | 2450 | 874 |
| Mid Range | 1.4/3/5 10 [1] | 20525 | 836.5 | 2525 | 881.5 |
| High Range | 1.4 | 20643 | 848.3 | 2643 | 893.3 |
|  | 3 | 20635 | 847.5 | 2635 | 892.5 |
|  | 5 | 20625 | 846.5 | 2625 | 891.5 |
|  | 10 [1] | 20600 | 844 | 2600 | 889 |
| NOTE 1: Bandwidth for which a relaxation of the specified UE receiver sensitivity requirement (TS 36.101 [27] Clause 7.3) is allowed. |  |  |  |  |  |

##### 4.3.1.1.5A	FDD reference test frequencies for CA in operating band 5

Table 4.3.1.1.5A-1: Test frequencies for CA_5B

| Range | CC-Combo / NRB_agg [RB] |  |  |  |  | CC1 Note1 |  |  |  |  |  |  |  | CC2 Note1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL |  | fUL [MHz] |  | NDL |  | fDL [MHz] |  | BW [RB] |  | NUL | fUL [MHz] | NDL | fDL [MHz] |
| Low | 15+25 | 15 | 20416 |  | 825.6 |  | 2416 |  | 870.6 |  | 25 |  | 20455 | 829.5 | 2455 | 874.5 |
|  |  | 25 | 20425 |  | 826.5 |  | 2425 |  | 871.5 |  | 15 |  | 20464 | 830.4 | 2464 | 875.4 |
|  | 25+50 | 25 | 20428 |  | 826.8 |  | 2428 |  | 871.8 |  | 50 |  | 20500 | 834 | 2500 | 879 |
|  | 50+25 | 50 | 20450 |  | 829 |  | 2450 |  | 874 |  | 25 |  | 20522 | 836.2 | 2522 | 881.2 |
|  | 50+50 | 50 | 20450 |  | 829 |  | 2450 |  | 874 |  | 50 |  | 20549 | 838.9 | 2549 | 883.9 |
| Mid | 15+25 | 15 | 20501 |  | 834.1 |  | 2501 |  | 879.1 |  | 25 |  | 20540 | 838.0 | 2540 | 883.0 |
|  |  | 25 | 20510 |  | 835.0 |  | 2510 |  | 880.0 |  | 15 |  | 20549 | 838.9 | 2549 | 883.9 |
|  | 25+50 | 25 | 20478 |  | 831.8 |  | 2478 |  | 876.8 |  | 50 |  | 20550 | 839 | 2550 | 884 |
|  | 50+25 | 50 | 20500 |  | 834 |  | 2500 |  | 879 |  | 25 |  | 20572 | 841.2 | 2572 | 886.2 |
|  | 50+50 | 50 | 20476 |  | 831.6 |  | 2476 |  | 876.6 |  | 50 |  | 20575 | 841.5 | 2575 | 886.5 |
| High | 15+25 | 15 | 20586 |  | 842.6 |  | 2586 |  | 887.6 |  | 25 |  | 20625 | 846.5 | 2625 | 891.5 |
|  |  | 25 | 20595 |  | 843.5 |  | 2595 |  | 888.5 |  | 15 |  | 20634 | 847.4 | 2634 | 892.4 |
|  | 25+50 | 25 | 20528 |  | 836.8 |  | 2528 |  | 881.8 |  | 50 |  | 20600 | 844 | 2600 | 889 |
|  | 50+25 | 50 | 20550 |  | 839 |  | 2550 |  | 884 |  | 25 |  | 20622 | 846.2 | 2622 | 891.2 |
|  | 50+50 | 50 | 20501 |  | 834.1 |  | 2501 |  | 879.1 |  | 50 |  | 20600 | 844 | 2600 | 889 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.5A-2: Test frequencies for CA_5A-5A

| Test Frequency ID | CC-Combo / NRB_agg [RB] |  |  | CC1 Note1 |  |  | Wgap [MHz] |  |  | CC2 Note1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL | fUL [MHz] | NDL | fDL [MHz] |  | BW [RB] | NUL | fUL [MHz] | NDL | fDL [MHz] |
| Max WGap | 25+25 | 25 | 20425 | 826.5 | 2425 | 871.5 | 15 | 25 | 20625 | 846.5 | 2625 | 891.5 |
|  | 25+50 | 25 | 20425 | 826.5 | 2425 | 871.5 | 10 | 50 | 20600 | 844 | 2600 | 889 |
|  |  | 50 | 20450 | 829 | 2450 | 874 | 10 | 25 | 20625 | 846.5 | 2625 | 891.5 |
|  | 50+50 | 50 | 20450 | 829 | 2450 | 874 | 5 | 50 | 20600 | 844 | 2600 | 889 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |  |  |

##### 4.3.1.1.6	FDD reference test frequencies for operating band 6

Table 4.3.1.1.6-1: Test frequencies for E-UTRA channel bandwidth for operating band 6

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 5 | 20675 | 832.5 | 2675 | 877.5 |
|  | 10 [1] | 20700 | 835 | 2700 | 880 |
| Mid Range | 5 10 [1] | 20700 | 835 | 2700 | 880 |
| High Range | 5 | 20725 | 837.5 | 2725 | 882.5 |
|  | 10 [1] | 20700 | 835 | 2700 | 880 |
| NOTE 1: Bandwidth for which a relaxation of the specified UE receiver sensitivity requirement (TS 36.101 [27] Clause 7.3) is allowed. |  |  |  |  |  |

NOTE:	For Band 6 testing, the Mobile Country Code shall be set to (MCC = '442/443').

##### 4.3.1.1.7	FDD reference test frequencies for operating band 7

Table 4.3.1.1.7-1: Test frequencies for E-UTRA channel bandwidth for operating band 7

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 5 | 20775 | 2502.5 | 2775 | 2622.5 |
|  | 10 | 20800 | 2505 | 2800 | 2625 |
|  | 15 | 20825 | 2507.5 | 2825 | 2627.5 |
|  | 20 [1] | 20850 | 2510 | 2850 | 2630 |
| Mid Range | 5/10/15 20 [1] | 21100 | 2535 | 3100 | 2655 |
| High Range | 5 | 21425 | 2567.5 | 3425 | 2687.5 |
|  | 10 | 21400 | 2565 | 3400 | 2685 |
|  | 15 | 21375 | 2562.5 | 3375 | 2682.5 |
|  | 20 [1] | 21350 | 2560 | 3350 | 2680 |
| NOTE 1: Bandwidth for which a relaxation of the specified UE receiver sensitivity requirement (TS 36.101 [27] Clause 7.3) is allowed. |  |  |  |  |  |

##### 4.3.1.1.7A	FDD reference test frequencies for CA in operating band 7

Table 4.3.1.1.7A-1: Test frequencies for CA_7C

| Range | CC-Combo / NRB_agg [RB] |  |  |  |  | CC1 Note1 |  |  |  |  |  |  |  | CC2 Note1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL |  | fUL [MHz] |  | NDL |  | fDL [MHz] |  | BW [RB] | NUL |  | fUL [MHz] | NDL |  | fDL [MHz] |  |  |
| Low | 50+100 | 50 | 20805 |  | 2505.5 |  | 2805 |  | 2625.5 |  | 100 | 20949 |  | 2519.9 | 2949 |  | 2639.9 |  |  |
|  |  | 100 | 20850 |  | 2510 |  | 2850 |  | 2630 |  | 50 | 20994 |  | 2524.4 | 2994 |  | 2644.4 |  |  |
|  | 75+50 | 75 | 20825 |  | 2507.5 |  | 2825 |  | 2627.5 |  | 50 | 20945 |  | 2519.5 |  | 2945 |  | 2639.5 |  |
|  | 75+75 | 75 | 20825 |  | 2507.5 |  | 2825 |  | 2627.5 |  | 75 | 20975 |  | 2522.5 | 2975 |  | 2642.5 |  |  |
|  | 75+100 | 75 | 20828 |  | 2507.8 |  | 2828 |  | 2627.8 |  | 100 | 20999 |  | 2524.9 | 2999 |  | 2644.9 |  |  |
|  |  | 100 | 20850 |  | 2510 |  | 2850 |  | 2630 |  | 75 | 21021 |  | 2527.1 | 3021 |  | 2647.1 |  |  |
|  | 100+100 | 100 | 20850 |  | 2510 |  | 2850 |  | 2630 |  | 100 | 21048 |  | 2529.8 | 3048 |  | 2649.8 |  |  |
| Mid | 50+100 | 50 | 21006 |  | 2525.6 |  | 3006 |  | 2645.6 |  | 100 | 21150 |  | 2540 | 3150 |  | 2660 |  |  |
|  |  | 100 | 21051 |  | 2530.1 |  | 3051 |  | 2650.1 |  | 50 | 21195 |  | 2544.5 | 3195 |  | 2664.5 |  |  |
|  | 75+50 | 75 | 21051 |  | 2530.1 |  | 3051 |  | 2650.1 |  | 50 | 21171 |  | 2542.1 |  | 3171 |  | 2662.1 |  |
|  | 75+75 | 75 | 21025 |  | 2527.5 |  | 3025 |  | 2647.5 |  | 75 | 21175 |  | 2542.5 | 3175 |  | 2662.5 |  |  |
|  | 75+100 | 75 | 21003 |  | 2525.3 |  | 3003 |  | 2645.3 |  | 100 | 21174 |  | 2542.4 | 3174 |  | 2662.4 |  |  |
|  |  | 100 | 21026 |  | 2527.6 |  | 3026 |  | 2647.6 |  | 75 | 21197 |  | 2544.7 | 3197 |  | 2664.7 |  |  |
|  | 100+100 | 100 | 21001 |  | 2525.1 |  | 3001 |  | 2645.1 |  | 100 | 21199 |  | 2544.9 | 3199 |  | 2664.9 |  |  |
| High | 50+100 | 50 | 21206 |  | 2545.6 |  | 3206 |  | 2665.6 |  | 100 | 21350 |  | 2560 | 3350 |  | 2680 |  |  |
|  |  | 100 | 21251 |  | 2550.1 |  | 3251 |  | 2670.1 |  | 50 | 21395 |  | 2564.5 | 3395 |  | 2684.5 |  |  |
|  | 75+50 | 75 | 21277 |  | 2552.7 |  | 3277 |  | 2672.7 |  | 50 | 21397 |  | 2564.7 |  | 3397 |  | 2684.7 |  |
|  | 75+75 | 75 | 21225 |  | 2547.5 |  | 3225 |  | 2667.5 |  | 75 | 21375 |  | 2562.5 | 3375 |  | 2682.5 |  |  |
|  | 75+100 | 75 | 21179 |  | 2542.9 |  | 3179 |  | 2662.9 |  | 100 | 21350 |  | 2560 | 3350 |  | 2680 |  |  |
|  |  | 100 | 21201 |  | 2545.1 |  | 3201 |  | 2665.1 |  | 75 | 21372 |  | 2562.2 | 3372 |  | 2682.2 |  |  |
|  | 100+100 | 100 | 21152 |  | 2540.2 |  | 3152 |  | 2660.2 |  | 100 | 21350 |  | 2560 | 3350 |  | 2680 |  |  |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.7A-2 : Test frequencies for CA_7B

| Range | CC-Combo / NRB_agg [RB] |  |  | CC1 Note1 |  |  |  |  | CC2 Note1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL | fUL [MHz] | NDL | fDL [MHz] | BW [RB] | NUL | fUL [MHz] | NDL | fDL [MHz] |
| Low | 75+25 | 75 | 20825 | 2507.5 | 2825 | 2627.5 | 25 | 20918 | 2516.8 | 2918 | 2636.8 |
| Mid | 75+25 | 75 | 21076 | 2532.6 | 3076 | 2652.6 | 25 | 21169 | 2541.9 | 3169 | 2661.9 |
| High | 75+25 | 75 | 21327 | 2557.7 | 3327 | 2677.7 | 25 | 21420 | 2567.0 | 3420 | 2687.0 |
| Note 1: Carriers in increasing frequency order |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.7A-2: Test frequencies for CA_7A-7A

| Test Frequency ID | CC-Combo / NRB_agg [RB] |  |  | CC1 Note1 |  |  | Wgap [MHz] |  |  | CC2 Note1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL | fUL [MHz] | NDL | fDL [MHz] |  | BW [RB] | NUL | fUL [MHz] | NDL | fDL [MHz] |
| Max WGap | 25+25 | 25 | 20775 | 2502.5 | 2775 | 2622.5 | 60 | 25 | 21425 | 2567.5 | 3425 | 2687.5 |
|  | 25+50 | 25 | 20775 | 2502.5 | 2775 | 2622.5 | 55 | 50 | 21400 | 2565 | 3400 | 2685 |
|  |  | 50 | 20800 | 2505 | 2800 | 2625 | 55 | 25 | 21425 | 2567.5 | 3425 | 2687.5 |
|  | 25+75 | 25 | 20775 | 2502.5 | 2775 | 2622.5 | 50 | 75 | 21375 | 2562.5 | 3375 | 2682.5 |
|  |  | 75 | 20825 | 2507.5 | 2825 | 2627.5 | 50 | 25 | 21425 | 2567.5 | 3425 | 2687.5 |
|  | 50+50 | 50 | 20800 | 2505 | 2800 | 2625 | 50 | 50 | 21400 | 2565 | 3400 | 2685 |
|  | 25+100 | 25 | 20775 | 2502.5 | 2775 | 2622.5 | 45 | 100 | 21350 | 2560 | 3350 | 2680 |
|  |  | 100 | 20850 | 2510 | 2850 | 2630 | 45 | 25 | 21425 | 2567.5 | 3425 | 2687.5 |
|  | 50+75 | 50 | 20800 | 2505 | 2800 | 2625 | 45 | 75 | 21375 | 2562.5 | 3375 | 2682.5 |
|  |  | 75 | 20825 | 2507.5 | 2825 | 2627.5 | 45 | 50 | 21400 | 2565 | 3400 | 2685 |
|  | 50+100 | 50 | 20800 | 2505 | 2800 | 2625 | 40 | 100 | 21350 | 2560 | 3350 | 2680 |
|  |  | 100 | 20850 | 2510 | 2850 | 2630 | 40 | 50 | 21400 | 2565 | 3400 | 2685 |
|  | 75+75 | 75 | 20825 | 2507.5 | 2825 | 2627.5 | 40 | 75 | 21375 | 2562.5 | 3375 | 2682.5 |
|  | 75+100 | 75 | 20825 | 2507.5 | 2825 | 2627.5 | 35 | 100 | 21350 | 2560 | 3350 | 2680 |
|  |  | 100 | 20850 | 2510 | 2850 | 2630 | 35 | 75 | 21375 | 2562.5 | 3375 | 2682.5 |
|  | 100+100 | 100 | 20850 | 2510 | 2850 | 2630 | 30 | 100 | 21350 | 2560 | 3350 | 2680 |
| Refsens2 | 75+100 | 75 | 21025 | 2527.5 | 3025 | 2647.5 | 15 | 100 | 21350 | 2560 | 3350 | 2680 |
|  | 100+100 | 100 | 21000 | 2525 | 3000 | 2645 | 15 | 100 | 21350 | 2560 | 3350 | 2680 |
|  | 25+100 | 25 | 20975 | 2522.5 | 2975 | 2642.5 | 25 | 100 | 21350 | 2560 | 3350 | 2680 |
|  | 25+50 | 25 | 21025 | 2527.5 | 3025 | 2647.5 | 30 | 50 | 21400 | 2565 | 3400 | 2685 |
|  | 50+100 | 50 | 21000 | 2525 | 3000 | 2645 | 20 | 100 | 21350 | 2560 | 3350 | 2680 |
|  |  | 100 | 21100 | 2535 | 3100 | 2655 | 15 | 50 | 21400 | 2565 | 3400 | 2685 |
| Note 1: Carriers in increasing frequency order. Note 2: Test point derived with regard to REFSENS requirements. |  |  |  |  |  |  |  |  |  |  |  |  |

##### 4.3.1.1.8	FDD reference test frequencies for operating band 8

Table 4.3.1.1.8-1: Test frequencies for E-UTRA channel bandwidth for operating band 8

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 1.4 | 21457 | 880.7 | 3457 | 925.7 |
|  | 3 | 21465 | 881.5 | 3465 | 926.5 |
|  | 5 | 21475 | 882.5 | 3475 | 927.5 |
|  | 10 [1] | 21500 | 885 | 3500 | 930 |
| Mid Range | 1.4/3/5 10 [1] | 21625 | 897.5 | 3625 | 942.5 |
| High Range | 1.4 | 21793 | 914.3 | 3793 | 959.3 |
|  | 3 | 21785 | 913.5 | 3785 | 958.5 |
|  | 5 | 21775 | 912.5 | 3775 | 957.5 |
|  | 10 [1] | 21750 | 910 | 3750 | 955 |
| NOTE 1: Bandwidth for which a relaxation of the specified UE receiver sensitivity requirement (TS 36.101 [27] Clause 7.3) is allowed. |  |  |  |  |  |

##### 4.3.1.1.8A	FDD reference test frequencies for CA in operating band 8

Table 4.3.1.1. 8 A-1: Test frequencies for CA_8B

| Range | CC-Combo / NRB_agg [RB] |  |  |  |  | CC1 Note1 |  |  |  |  |  |  |  | CC2 Note1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL |  | fUL [MHz] |  | NDL |  | fDL [MHz] |  | BW [RB] |  | NUL | fUL [MHz] | NDL | fDL [MHz] |
| Low | 25+50 | 25 | 21478 |  | 882.8 |  | 3478 |  | 927.8 |  | 50 |  | 21550 | 890 | 3550 | 935 |
|  | 50+25 | 50 | 21500 |  | 885 |  | 3500 |  | 930 |  | 25 |  | 21572 | 892.2 | 3572 | 937.2 |
|  | 50+50 | 50 | 21500 |  | 885 |  | 3500 |  | 930 |  | 50 |  | 21599 | 894.9 | 3599 | 939.9 |
| Mid | 25+50 | 25 | 21578 |  | 892.8 |  | 3578 |  | 937.8 |  | 50 |  | 21650 | 900.0 | 3650 | 945.0 |
|  | 50+25 | 50 | 21600 |  | 895.0 |  | 3600 |  | 940.0 |  | 25 |  | 21672 | 902.2 | 3672 | 947.2 |
|  | 50+50 | 50 | 21576 |  | 892.6 |  | 3576 |  | 937.6 |  | 50 |  | 21675 | 902.5 | 3675 | 947.5 |
| High | 25+50 | 25 | 21678 |  | 902.8 |  | 3678 |  | 947.8 |  | 50 |  | 21750 | 910 | 3750 | 955 |
|  | 50+25 | 50 | 21700 |  | 905 |  | 3700 |  | 950 |  | 25 |  | 21772 | 912.2 | 3772 | 957.2 |
|  | 50+50 | 50 | 21651 |  | 900.1 |  | 3651 |  | 945.1 |  | 50 |  | 21750 | 910 | 3750 | 955 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### 4.3.1.1.9	FDD reference test frequencies for operating band 9

Table 4.3.1.1.9-1: Test frequencies for E-UTRA channel bandwidth for operating band 9

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 5 | 21825 | 1752.4 | 3825 | 1847.4 |
|  | 10 | 21850 | 1754.9 | 3850 | 1849.9 |
|  | 15 [1] | 21875 | 1757.4 | 3875 | 1852.4 |
|  | 20 [1] | 21900 | 1759.9 | 3900 | 1854.9 |
| Mid Range | 5/10 15 [1]/20 [1] | 21975 | 1767.4 | 3975 | 1862.4 |
| High Range | 5 | 22125 | 1782.4 | 4125 | 1877.4 |
|  | 10 | 22100 | 1779.9 | 4100 | 1874.9 |
|  | 15 [1] | 22075 | 1777.4 | 4075 | 1872.4 |
|  | 20 [1] | 22050 | 1774.9 | 4050 | 1869.9 |
| NOTE 1: Bandwidth for which a relaxation of the specified UE receiver sensitivity requirement (TS 36.101 [27] Clause 7.3) is allowed. |  |  |  |  |  |

##### 4.3.1.1.10	FDD reference test frequencies for operating band 10

Table 4.3.1.1.10-1: Test frequencies for E-UTRA channel bandwidth for operating band 10

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 5 | 22175 | 1712.5 | 4175 | 2112.5 |
|  | 10 | 22200 | 1715 | 4200 | 2115 |
|  | 15 | 22225 | 1717.5 | 4225 | 2117.5 |
|  | 20 | 22250 | 1720 | 4250 | 2120 |
| Mid Range | 5/10/15/20 | 22450 | 1740 | 4450 | 2140 |
| High Range | 5 | 22725 | 1767.5 | 4725 | 2167.5 |
|  | 10 | 22700 | 1765 | 4700 | 2165 |
|  | 15 | 22675 | 1762.5 | 4675 | 2162.5 |
|  | 20 | 22650 | 1760 | 4650 | 2160 |

##### 4.3.1.1.11	FDD reference test frequencies for operating band 11

Table 4.3.1.1.11-1: Test frequencies for E-UTRA channel bandwidth for operating band 11

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 5 | 22775 | 1430.4 | 4775 | 1478.4 |
|  | 10 [1] | 22800 | 1432.9 | 4800 | 1480.9 |
| Mid Range | 5 / 10 [1] | 22850 | 1437.9 | 4850 | 1485.9 |
| High Range | 5 | 22925 | 1445.4 | 4925 | 1493.4 |
|  | 10 [1] | 22900 | 1442.9 | 4900 | 1490.9 |
| NOTE 1: Bandwidth for which a relaxation of the specified UE receiver sensitivity requirement (TS 36.101 [27] Clause 7.3) is allowed. |  |  |  |  |  |

##### 4.3.1.1.12	FDD reference test frequencies for operating band 12

Table 4.3.1.1.12-1: Test frequencies for E-UTRA channel bandwidth for operating band 12

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 1.4 | 23017 | 699.7 | 5017 | 729.7 |
|  | 3 | 23025 | 700.5 | 5025 | 730.5 |
|  | 5 [1] | 23035 | 701.5 | 5035 | 731.5 |
|  | 10 [1] | 23060 | 704 | 5060 | 734 |
| Mid Range | 1.4/3 5 [1]/10 [1] | 23095 | 707.5 | 5095 | 737.5 |
| High Range | 1.4 | 23173 | 715.3 | 5173 | 745.3 |
|  | 3 | 23165 | 714.5 | 5165 | 744.5 |
|  | 5 [1] | 23155 | 713.5 | 5155 | 743.5 |
|  | 10 [1] | 23130 | 711 | 5130 | 741 |
| NOTE 1: Bandwidth for which a relaxation of the specified UE receiver sensitivity requirement (TS 36.101 [27] Clause 7.3) is allowed. |  |  |  |  |  |

##### 4.3.1.1.12A	FDD reference test frequencies for CA in operating band 12

Table 4.3.1.1.12A-1: Test frequencies for CA_12B

| Range | CC-Combo / NRB_agg [RB] |  |  | CC1 Note1 |  |  |  |  | CC2 Note1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL | fUL [MHz] | NDL | fDL [MHz] | BW [RB] | NUL | fUL [MHz] | NDL | fDL [MHz] |
| Low | 25+25 | 25 | 23035 | 701.5 | 5035 | 731.5 | 25 | 23083 | 706.3 | 5083 | 736.3 |
|  | 25+50 | 25 | 23038 | 701.8 | 5038 | 731.8 | 50 | 23110 | 709 | 5110 | 739 |
| Mid | 25+25 | 25 | 23071 | 705.1 | 5071 | 735.1 | 25 | 23119 | 709.9 | 5119 | 739.9 |
|  | 25+50 | 25 | 23048 | 702.8 | 5048 | 732.8 | 50 | 23120 | 710 | 5120 | 740 |
| High | 25+25 | 25 | 23107 | 708.7 | 5107 | 738.7 | 25 | 23155 | 713.5 | 5155 | 743.5 |
|  | 25+50 | 25 | 23058 | 703.8 | 5058 | 733.8 | 50 | 23130 | 711 | 5130 | 741 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.12A-2: Test frequencies for CA_12A-12A

| Test Frequency ID | CC-Combo / NRB_agg [RB] |  |  | CC1 Note1 |  |  | Wgap [MHz] |  |  | CC2 Note1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL | fUL [MHz] | NDL | fDL [MHz] |  | BW [RB] | NUL | fUL [MHz] | NDL | fDL [MHz] |
| Max WGap | 25+25 | 25 | 23035 | 701.5 | 5035 | 731.5 | 7 | 25 | 23155 | 713.5 | 5155 | 743.5 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |  |  |

##### 4.3.1.1.13	FDD reference test frequencies for operating band 13

Table 4.3.1.1.13-1: Test frequencies for E-UTRA channel bandwidth for operating band 13

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 5 [1] | 23205 | 779.5 | 5205 | 748.5 |
|  | 10 [1] | 23230 | 782 | 5230 | 751 |
| Mid Range | 5 [1]/10 [1] | 23230 | 782 | 5230 | 751 |
| High Range | 5 [1] | 23255 | 784.5 | 5255 | 753.5 |
|  | 10 [1] | 23230 | 782 | 5230 | 751 |
| NOTE 1: Bandwidth for which a relaxation of the specified UE receiver sensitivity requirement (TS 36.101 [27] Clause 7.3) is allowed. |  |  |  |  |  |

##### 4.3.1.1.14	FDD reference test frequencies for operating band 14

Table 4.3.1.1.14-1: Test frequencies for E-UTRA channel bandwidth for operating band 14

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 5 [1] | 23305 | 790.5 | 5305 | 760.5 |
|  | 10 [1] | 23330 | 793 | 5330 | 763 |
| Mid Range | 5 [1]/10 [1] | 23330 | 793 | 5330 | 763 |
| High Range | 5 [1] | 23355 | 795.5 | 5355 | 765.5 |
|  | 10 [1] | 23330 | 793 | 5330 | 763 |
| NOTE 1: Bandwidth for which a relaxation of the specified UE receiver sensitivity requirement (TS 36.101 [27] Clause 7.3) is allowed. |  |  |  |  |  |

##### 4.3.1.1.15	FDD reference test frequencies for operating band 15

[FFS; not yet specified in TS 36.101]

##### 4.3.1.1.16	FDD reference test frequencies for operating band 16

[FFS; not yet specified in TS 36.101]

##### 4.3.1.1.17	FDD reference test frequencies for operating band 17

Table 4.3.1.1.17-1: Test frequencies for E-UTRA channel bandwidth for operating band 17

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 5 [1] | 23755 | 706.5 | 5755 | 736.5 |
|  | 10 [1] | 23780 | 709 | 5780 | 739 |
| Mid Range | 5 [1]/10 [1] | 23790 | 710 | 5790 | 740 |
| High Range | 5 [1] | 23825 | 713.5 | 5825 | 743.5 |
|  | 10 [1] | 23800 | 711 | 5800 | 741 |
| NOTE 1: Bandwidth for which a relaxation of the specified UE receiver sensitivity requirement (TS 36.101 [27] Clause 7.3) is allowed. |  |  |  |  |  |

##### 4.3.1.1.18	FDD reference test frequencies for operating band 18

Table 4.3.1.1.18-1: Test frequencies for E-UTRA channel bandwidth for operating band 18

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 5 | 23875 | 817.5 | 5875 | 862.5 |
|  | 10 [1] | 23900 | 820 | 5900 | 865 |
|  | 15 [1] | 23925 | 822.5 | 5925 | 867.5 |
| Mid Range | 5/10 [1]/15 [1] | 23925 | 822.5 | 5925 | 867.5 |
| High range | 5 | 23975 | 827.5 | 5975 | 872.5 |
|  | 10 [1] | 23950 | 825 | 5950 | 870 |
|  | 15 [1] | 23925 | 822.5 | 5925 | 867.5 |
| NOTE 1: Bandwidth for which a relaxation of the specified UE receiver sensitivity requirement (TS 36.101 [27] Clause 7.3) is allowed. |  |  |  |  |  |

##### 4.3.1.1.19	FDD reference test frequencies for operating band 19

Table 4.3.1.1.19-1: Test frequencies for E-UTRA channel bandwidth for operating band 19

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 5 | 24025 | 832.5 | 6025 | 877.5 |
|  | 10 [1] | 24050 | 835 | 6050 | 880 |
|  | 15 [1] | 24075 | 837.5 | 6075 | 882.5 |
| Mid Range | 5/10 [1]/15 [1] | 24075 | 837.5 | 6075 | 882.5 |
| High range | 5 | 24125 | 842.5 | 6125 | 887.5 |
|  | 10 [1] | 24100 | 840 | 6100 | 885 |
|  | 15 [1] | 24075 | 837.5 | 6075 | 882.5 |
| NOTE 1: Bandwidth for which a relaxation of the specified UE receiver sensitivity requirement (TS 36.101 [27] Clause 7.3) is allowed. |  |  |  |  |  |

##### 4.3.1.1.20	FDD reference test frequencies for operating band 20

Table 4.3.1.1.20-1: Test frequencies for E-UTRA channel bandwidth for operating band 20

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 5 | 24175 | 834.5 | 6175 | 793.5 |
|  | 10[1] | 24200 | 837 | 6200 | 796 |
|  | 15[1] | 24225 | 839.5 | 6225 | 798.5 |
|  | 20[1] | 24250 | 842 | 6250 | 801 |
| Mid Range | 5/10[1]/15[1]/20[1] | 24300 | 847 | 6300 | 806 |
| High range | 5 | 24425 | 859.5 | 6425 | 818.5 |
|  | 10[1] | 24400 | 857 | 6400 | 816 |
|  | 15[1] | 24375 | 854.5 | 6375 | 813.5 |
|  | 20[1] | 24350 | 852 | 6350 | 811 |
| NOTE 1: Bandwidth for which a relaxation of the specified UE receiver sensitivity requirement (TS 36.101 [27] Clause 7.3) is allowed. |  |  |  |  |  |

##### 4.3.1.1.21	FDD reference test frequencies for operating band 21

Table 4.3.1.1.21-1: Test frequencies for E-UTRA channel bandwidth for operating band 21

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 5 | 24475 | 1450.4 | 6475 | 1498.4 |
|  | 10 [1] | 24500 | 1452.9 | 6500 | 1500.9 |
|  | 15 [1] | 24525 | 1455.4 | 6525 | 1503.4 |
| Mid Range | 5/10 [1]/15 [1] | 24525 | 1455.4 | 6525 | 1503.4 |
| High range | 5 | 24575 | 1460.4 | 6575 | 1508.4 |
|  | 10 [1] | 24550 | 1457.9 | 6550 | 1505.9 |
|  | 15 [1] | 24525 | 1455.4 | 6525 | 1503.4 |
| NOTE 1: Bandwidth for which a relaxation of the specified UE receiver sensitivity requirement (TS 36.101 [27] Clause 7.3) is allowed. |  |  |  |  |  |

##### 4.3.1.1.22	FDD reference test frequencies for operating band 22

Table 4.3.1.1.22-1: Test frequencies for E-UTRA channel bandwidth for operating band 22

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 5 | 24625 | 3412.5 | 6625 | 3512.5 |
|  | 10 | 24650 | 3415 | 6650 | 3515 |
|  | 15 | 24675 | 3417.5 | 6675 | 3517.5 |
|  | 20 | 24700 | 3420 | 6700 | 3520 |
| Mid Range | 5/10/15/20 | 25000 | 3450 | 7000 | 3550 |
| High Range | 5 | 25375 | 3487.5 | 7375 | 3587.5 |
|  | 10 | 25350 | 3485 | 7350 | 3585 |
|  | 15 | 25325 | 3482.5 | 7325 | 3582.5 |
|  | 20 | 25300 | 3480 | 7300 | 3580 |

##### 4.3.1.1.23	FDD reference test frequencies for operating band 23

Table 4.3.1.1.23-1: Test frequencies for E-UTRA channel bandwidth for operating band 23

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 1.4 | 25507 | 2000.7 | 7507 | 2180.7 |
|  | 3 | 25515 | 2001.5 | 7515 | 2181.5 |
|  | 5 | 25525 | 2002.5 | 7525 | 2182.5 |
|  | 10 | 25550 | 2005 | 7550 | 2185 |
|  | 15 | 25575 | 2007.5 | 7575 | 2187.5 |
|  | 20 | 25600 | 2010 | 7600 | 2190 |
| Mid Range | 1.4/3 5 /10 / 15 / 20 | 25600 | 2010 | 7600 | 2190 |
| High Range | 1.4 | 25693 | 2019.3 | 7693 | 2199.3 |
|  | 3 | 25685 | 2018.5 | 7685 | 2198.5 |
|  | 5 | 25675 | 2017.5 | 7675 | 2197.5 |
|  | 10 | 25650 | 2015 | 7650 | 2195 |
|  | 15 | 25625 | 2012.5 | 7625 | 2192.5 |
|  | 20 | 25600 | 2010 | 7600 | 2190 |

##### 4.3.1.1.23A	FDD reference test frequencies for CA in operating band 23

Table 4.3.1.1.23A-1: Test frequencies for CA_23A-23A

| Test Frequency ID | CC-Combo / NRB_agg [RB] |  |  | CC1 Note1 |  |  | Wgap [MHz] |  |  | CC2 Note1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL | fUL [MHz] | NDL | fDL [MHz] |  | BW [RB] | NUL | fUL [MHz] | NDL | fDL [MHz] |
| Max WGap | 25+50 | 25 | 25525 | 2002.5 | 7525 | 2182.5 | 5 | 50 | 25650 | 2015 | 7650 | 2195 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.23A-2: Test frequencies for CA_23B

| Range | CC-Combo / NRB_agg [RB] |  |  |  |  | CC1 Note1 |  |  |  |  |  |  |  | CC2 Note1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL |  | fUL [MHz] |  | NDL |  | fDL [MHz] |  | BW [RB] | NUL |  | fUL [MHz] | NDL | fDL [MHz] |  |
| Low/Mid/High | 25+75 | 25 | 25531 |  | 2003.1 |  | 7531 |  | 2183.1 |  | 75 | 25624 |  | 2012.4 | 7624 | 2192.4 |  |
|  | 50+50 | 50 | 25551 |  | 2005.1 |  | 7551 |  | 2185.1 |  | 50 | 25650 |  | 2015 | 7650 | 2195 |  |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### 4.3.1.1.24	FDD reference test frequencies for operating band 24

Table 4.3.1.1.24-1: Test frequencies for E-UTRA channel bandwidth for operating band 24

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 5 | 25725 | 1629 | 7725 | 1527.5 |
|  | 10 | 25750 | 1631.5 | 7750 | 1530 |
| Mid Range | 5/10 | 25870 | 1643.5 | 7870 | 1542 |
| High Range | 5 | 26015 | 1658 | 8015 | 1556.5 |
|  | 10 | 25990 | 1655.5 | 7990 | 1554 |

##### 4.3.1.1.25	FDD reference test frequencies for operating band 25

Table 4.3.1.1.25-1: Test frequencies for E-UTRA channel bandwidth for operating band 25

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 1.4 | 26047 | 1850.7 | 8047 | 1930.7 |
|  | 3 | 26055 | 1851.5 | 8055 | 1931.5 |
|  | 5 | 26065 | 1852.5 | 8065 | 1932.5 |
|  | 10 | 26090 | 1855 | 8090 | 1935 |
|  | 15 [1] | 26115 | 1857.5 | 8115 | 1937.5 |
|  | 20 [1] | 26140 | 1860 | 8140 | 1940 |
| Mid Range | 1.4/3/5/10 15 [1]/20 [1] | 26365 | 1882,5 | 8365 | 1962.5 |
| High Range | 1.4 | 26683 | 1914.3 | 8683 | 1994.3 |
|  | 3 | 26675 | 1913.5 | 8675 | 1993.5 |
|  | 5 | 26665 | 1912.5 | 8665 | 1992.5 |
|  | 10 | 26640 | 1910 | 8640 | 1990 |
|  | 15 [1] | 26615 | 1907.5 | 8615 | 1987.5 |
|  | 20 [1] | 26590 | 1905 | 8590 | 1985 |
| NOTE 1: Bandwidth for which a relaxation of the specified UE receiver sensitivity requirement (TS 36.101 [27] Clause 7.3) is allowed. |  |  |  |  |  |

##### 4.3.1.1.25A	FDD reference test frequencies for CA in operating band 25

Table 4.3.1.1.25A-1: Test frequencies for CA_25A-25A

| Test Frequency ID | CC-Combo / NRB_agg [RB] |  |  | CC1 Note1 |  |  | Wgap [MHz] |  |  | CC2 Note1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL | fUL [MHz] | NDL | fDL [MHz] |  | BW [RB] | NUL | fUL [MHz] | NDL | fDL [MHz] |
| Max WGap | 25+25 | 25 | 26065 | 1852.5 | 8065 | 1932.5 | 55 | 25 | 26665 | 1912.5 | 8665 | 1992.5 |
|  | 25+50 | 25 | 26065 | 1852.5 | 8065 | 1932.5 | 50 | 50 | 26640 | 1910 | 8640 | 1990 |
|  |  | 50 | 26090 | 1855 | 8090 | 1935 | 50 | 25 | 26665 | 1912.5 | 8665 | 1992.5 |
|  | 50+50 | 50 | 26090 | 1855 | 8090 | 1935 | 45 | 50 | 26640 | 1910 | 8640 | 1990 |
|  | 25+100 | 25 | 26065 | 1852.5 | 8065 | 1932.5 | 40 | 100 | 26590 | 1905 | 8590 | 1985 |
|  |  | 100 | 26140 | 1860 | 8140 | 1940 | 40 | 25 | 26665 | 1912.5 | 8665 | 1992.5 |
|  | 50+75 | 50 | 26090 | 1855 | 8090 | 1935 | 40 | 75 | 26615 | 1907.5 | 8615 | 1987.5 |
|  |  | 75 | 26115 | 1857.5 | 8115 | 1937.5 | 40 | 50 | 26640 | 1910 | 8640 | 1990 |
|  | 50+100 | 50 | 26090 | 1855 | 8090 | 1935 | 35 | 100 | 26590 | 1905 | 8590 | 1985 |
|  |  | 100 | 26140 | 1860 | 8140 | 1940 | 35 | 50 | 26640 | 1910 | 8640 | 1990 |
|  | 75+75 | 75 | 26115 | 1857.5 | 8115 | 1937.5 | 35 | 75 | 26615 | 1907.5 | 8615 | 1987.5 |
|  | 75+100 | 75 | 26115 | 1857.5 | 8115 | 1937.5 | 30 | 100 | 26590 | 1905 | 8590 | 1985 |
|  |  | 100 | 26140 | 1860 | 8140 | 1940 | 30 | 75 | 26615 | 1907.5 | 8615 | 1987.5 |
|  | 100+100 | 100 | 26140 | 1860 | 8140 | 1940 | 25 | 100 | 26590 | 1905 | 8590 | 1985 |
| Refsens2 | 25+50 | 25 | 26415 | 1887.5 | 8415 | 1967.5 | 15 | 50 | 26640 | 1910 | 8640 | 1990 |
|  | 50+50 | 50 | 26440 | 1890 | 8440 | 1970 | 10 | 50 | 26640 | 1910 | 8640 | 1990 |
| Note 1: Carriers in increasing frequency order. Note 2: Test point derived with regard to REFSENS requirements. |  |  |  |  |  |  |  |  |  |  |  |  |

##### 4.3.1.1.26	FDD reference test frequencies for operating band 26

Table 4.3.1.1.26-1: Test frequencies for E-UTRA channel bandwidth for operating band 26

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 1.4 | 26697 | 814.7 | 8697 | 859.7 |
|  | 3 | 26705 | 815.5 | 8705 | 860.5 |
|  | 5 | 26715 | 816.5 | 8715 | 861.5 |
|  | 10[1] | 26740 | 819 | 8740 | 864 |
|  | 15 [1] | 26765 | 821.5 | 8765 | 866.5 |
| Mid Range | 1.4/3/5/10[1] 15 [1] | 26865 | 831.5 | 8865 | 876.5 |
| High Range | 1.4 | 27033 | 848.3 | 9033 | 893.3 |
|  | 3 | 27025 | 847.5 | 9025 | 892.5 |
|  | 5 | 27015 | 846.5 | 9015 | 891.5 |
|  | 10[1] | 26990 | 844 | 8990 | 889 |
|  | 15 [1] | 26965 | 841.5 | 8965 | 886.5 |
| NOTE 1: Bandwidth for which a relaxation of the specified UE receiver sensitivity requirement (TS 36.101 [27] Clause 7.3) is allowed. |  |  |  |  |  |

##### 4.3.1.1.27	FDD reference test frequencies for operating band 27

Table 4.3.1.1.27-1: Test frequencies for E-UTRA channel bandwidth for operating band 27

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 1.4 | 27047 | 807.7 | 9047 | 852.7 |
|  | 3 | 27055 | 808.5 | 9055 | 853.5 |
|  | 5 | 27065 | 809.5 | 9065 | 854.5 |
|  | 10 [1] | 27090 | 812 | 9090 | 857 |
| Mid Range | 1.4/3/5/10 [1] | 27125 | 815.5 | 9125 | 860.5 |
| High Range | 1.4 | 27203 | 823.3 | 9203 | 868.3 |
|  | 3 | 27195 | 822.5 | 9195 | 867.5 |
|  | 5 | 27185 | 821.5 | 9185 | 866.5 |
|  | 10 [1] | 27160 | 819 | 9160 | 864 |
| NOTE 1: Bandwidth for which a relaxation of the specified UE receiver sensitivity requirement (TS 36.101 [27] Clause 7.3) is allowed. |  |  |  |  |  |

##### 4.3.1.1.27A	FDD reference test frequencies for CA in operating band 27

Table 4.3.1.1.27A-1: Test frequencies for CA_27B

| Range | CC-Combo / NRB_agg [RB] |  |  |  |  | CC1 Note1 |  |  |  |  |  |  |  | CC2 Note1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL |  | fUL [MHz] |  | NDL |  | fDL [MHz] |  | BW [RB] |  | NUL | fUL [MHz] | NDL | fDL [MHz] |
| Low | 6+25 | 6 | 27048 |  | 807.8 |  | 9048 |  | 852.8 |  | 25 |  | 27078 | 810.8 | 9078 | 855.8 |
|  | 15+25 | 15 | 27056 |  | 808.6 |  | 9056 |  | 853.6 |  | 25 |  | 27095 | 812.5 | 9095 | 857.5 |
|  | 25+25 | 25 | 27065 |  | 809.5 |  | 9065 |  | 854.5 |  | 25 |  | 27113 | 814.3 | 9113 | 859.3 |
|  | 6+50 | 6 | 27051 |  | 808.1 |  | 9051 |  | 853.1 |  | 50 |  | 27102 | 813.2 | 9102 | 858.2 |
|  | 15+50 | 15 | 27059 |  | 808.9 |  | 9059 |  | 853.9 |  | 50 |  | 27119 | 814.9 | 9119 | 859.9 |
| Mid | 6+25 | 6 | 27101 |  | 813.1 |  | 9101 |  | 858.1 |  | 25 |  | 27131 | 816.1 | 9131 | 861.1 |
|  | 15+25 | 15 | 27101 |  | 813.1 |  | 9101 |  | 858.1 |  | 25 |  | 27140 | 817.0 | 9140 | 862.0 |
|  | 25+25 | 25 | 27101 |  | 813.1 |  | 9101 |  | 858.1 |  | 25 |  | 27149 | 817.9 | 9149 | 862.9 |
|  | 6+50 | 6 | 27080 |  | 811.0 |  | 9080 |  | 856.0 |  | 50 |  | 27131 | 816.1 | 9131 | 861.1 |
|  | 15+50 | 15 | 27079 |  | 810.9 |  | 9079 |  | 855.9 |  | 50 |  | 27139 | 816.9 | 9139 | 861.9 |
| High | 6+25 | 6 | 27155 |  | 818.5 |  | 9155 |  | 863.5 |  | 25 |  | 27185 | 821.5 | 9185 | 866.5 |
|  | 15+25 | 15 | 27146 |  | 817.6 |  | 9146 |  | 862.6 |  | 25 |  | 27185 | 821.5 | 9185 | 866.5 |
|  | 25+25 | 25 | 27137 |  | 816.7 |  | 9137 |  | 861.7 |  | 25 |  | 27185 | 821.5 | 9185 | 866.5 |
|  | 6+50 | 6 | 27109 |  | 813.9 |  | 9109 |  | 858.9 |  | 50 |  | 27160 | 819.0 | 9160 | 864.0 |
|  | 15+50 | 15 | 27100 |  | 813.0 |  | 9100 |  | 858.0 |  | 50 |  | 27160 | 819.0 | 9160 | 864.0 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### 4.3.1.1.28	FDD reference test frequencies for operating band 28

Table 4.3.1.1.28-1: Test frequencies for E-UTRA channel bandwidth for operating band 28

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 3 | 27225 | 704.5 | 9225 | 759.5 |
|  | 5 | 27235 | 705.5 | 9235 | 760.5 |
|  | 10[1] | 27260 | 708 | 9260 | 763 |
|  | 15[1] | 27285 | 710.5 | 9285 | 765.5 |
|  | 20[1] | 27310 | 713 | 9310 | 768 |
| Mid Range | 3 | 27375 | 719.5 | 9375 | 774.5 |
|  | 5 | 27385 | 720.5 | 9385 | 775.5 |
|  | 10[1] | 27410 | 723 | 9410 | 778 |
|  | 15[1] | 27435 | 725.5 | 9435 | 780.5 |
|  | 20[1,2] | 27460 | 728 | 9460 | 783 |
| High Range | 3 | 27645 | 746.5 | 9645 | 801.5 |
|  | 5 | 27635 | 745.5 | 9635 | 800.5 |
|  | 10[1] | 27610 | 743 | 9610 | 798 |
|  | 15[1] | 27585 | 740.5 | 9585 | 795.5 |
|  | 20[1] | 27560 | 738 | 9560 | 793 |
| NOTE 1: Bandwidth for which a relaxation of the specified UE receiver sensitivity requirement (TS 36.101 [27] Clause 7.3) is allowed. NOTE 2: Mid Range for 20 MHz moved due to note 2 in Table 5.6.1-1 of TS 36.101 [27]. NOTE 3: For CA_18A-28A and CA_1A-18A-28A use test frequencies in Table 4.3.1.1.28-2. NOTE 4: For CA_19A-28A and CA_1A-19A-28A use test frequencies in Table 4.3.1.1.28-3. |  |  |  |  |  |

Table 4.3.1.1.28-2: Test frequencies for E-UTRA channel bandwidth for operating band 28 in CA_18A-28A and CA_1A-18A-28A

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 5 | 27235 | 705.5 | 9235 | 760.5 |
|  | 10[1] | 27260 | 708 | 9260 | 763 |
| Mid Range | 5/10[1] | 27360 | 718 | 9360 | 773 |
| High Range | 5 | 27485 | 730.5 | 9485 | 785.5 |
|  | 10[1] | 27460 | 728 | 9460 | 783 |
| NOTE 1: Bandwidth for which a relaxation of the specified UE receiver sensitivity requirement (TS 36.101 [27] Clause 7.3) is allowed. |  |  |  |  |  |

Table 4.3.1.1.28-3: Test frequencies for E-UTRA channel bandwidth for operating band 28 in CA_19A-28A and CA_1A-19A-28A

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 5 | 27385 | 720.5 | 9385 | 775.5 |
|  | 101 | 27410 | 723 | 9410 | 778 |
| Mid Range | 5/101 | 27510 | 733 | 9510 | 788 |
| High Range | 5 | 27635 | 745.5 | 9635 | 800.5 |
|  | 101 | 27610 | 743 | 9610 | 798 |
| NOTE 1: Bandwidth for which a relaxation of the specified UE receiver sensitivity requirement (TS 36.101 [27] Clause 7.3) is allowed. |  |  |  |  |  |

##### 4.3.1.1.29	FDD reference test frequencies for CA in operating band 29

Table 4.3.1.1.29-1: Test frequencies for E-UTRA channel bandwidth for operating band 29

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 3 | NA[1] | NA | 9675 | 718.5 |
|  | 5 | NA[1] | NA | 9685 | 719.5 |
|  | 10 | NA[1] | NA | 9710 | 722 |
| Mid Range | 3 5 /10 | NA[1] | NA | 9715 | 722.5 |
| High Range | 3 | NA[1] | NA | 9755 | 726.5 |
|  | 5 | NA[1] | NA | 9745 | 725.5 |
|  | 10 | NA[1] | NA | 9720 | 723 |
| NOTE 1: Restricted to E-UTRA operation when carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. |  |  |  |  |  |

##### 4.3.1.1.30	FDD reference test frequencies for operating band 30

Table 4.3.1.1.30-1: Test frequencies for E-UTRA channel bandwidth for operating band 30

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] |  | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 5 | 27685 | 2307.5 |  | 9795 | 2352.5 |
|  | 10 | 27710 | 2310 |  | 9820 | 2355 |
| Mid Range | 5/10 | 27710 | 2310 |  | 9820 | 2355 |
| High Range | 5 | 27735 | 2312.5 |  | 9845 | 2357.5 |
|  | 10 | 27710 | 2310 |  | 9820 | 2355 |
| Note 1: The uplink transmission is not allowed at this band for the UE with the externally vehicle-mounted antennas. |  |  |  |  |

##### 4.3.1.1.31	FDD reference test frequencies for operating band 31

Table 4.3.1.1.31-1: Test frequencies for E-UTRA channel bandwidth for operating band 31

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 1.4 | 27767 | 453.2 | 9877 | 463.2 |
|  | 3[1] | 27775 | 454 | 9885 | 464 |
|  | 5[1] | 27785 | 455 | 9895 | 465 |
| Mid Range | 1.4/3[1]/ 5[1] | 27785 | 455 | 9895 | 465 |
| High Range | 1.4 | 27803 | 456.8 | 9913 | 466.8 |
|  | 3[1] | 27795 | 456 | 9905 | 466 |
|  | 5[1] | 27785 | 455 | 9895 | 465 |
| NOTE 1: Bandwidth for which a relaxation of the specified UE receiver sensitivity requirement (TS 36.101 [27] Clause 7.3) is allowed. |  |  |  |  |  |

##### 4.3.1.1.32	FDD reference test frequencies for CA in operating band 32

Table 4.3.1.1.32-1: Test frequencies for E-UTRA channel bandwidth for operating band 32

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 5 | NA[1] | NA | 9945 | 1454.5 |
|  | 10 | NA[1] | NA | 9970 | 1457 |
|  | 15 | NA[1] | NA | 9995 | 1459.5 |
|  | 20 | NA[1] | NA | 10020 | 1462 |
| Mid Range | 5/10/15/20 | NA[1] | NA | 10140 | 1474 |
| High Range | 5 | NA[1] | NA | 10335 | 1493.5 |
|  | 10 | NA[1] | NA | 10310 | 1491 |
|  | 15 | NA[1] | NA | 10285 | 1488.5 |
|  | 20 | NA[1] | NA | 10260 | 1486 |
| NOTE 1: Restricted to E-UTRA operation when carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. |  |  |  |  |  |

##### 4.3.1.1.33 to 4.3.1.1.64	Void

Note:	Sub-clauses 4.3.1.1.33 to 4.3.1.1.64 are marked as void as bands 33 to 64 are TDD bands. TDD bands are specified in sub-clause 4.3.1.2.

##### 4.3.1.1.65	FDD reference test frequencies for operating band 65

Table 4.3.1.1.65-1: Test frequencies for E-UTRA channel bandwidth for operating band 65

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 1.4 | 131079 | 1920.7 | 65543 | 2110.7 |
|  | 3 | 131087 | 1921.5 | 65551 | 2111.5 |
|  | 5 | 131097 | 1922.5 | 65561 | 2112.5 |
|  | 10 | 131122 | 1925 | 65586 | 2115 |
|  | 15 | 131147 | 1927.5 | 65611 | 2117.5 |
|  | 20 | 131172 | 1930 | 65636 | 2120 |
| Mid Range | 1.4/3/5/10/15/20 | 131522 | 1965 | 65986 | 2155 |
| High Range | 1.4 | 131965 | 2009.3 | 66429 | 2199.3 |
|  | 3 | 131957 | 2008.5 | 66421 | 2198.5 |
|  | 5 | 131947 | 2007.5 | 66411 | 2197.5 |
|  | 10 | 131922 | 2005 | 66386 | 2195 |
|  | 15 | 131897 | 2002.5 | 66361 | 2192.5 |
|  | 20 | 131872 | 2000 | 66336 | 2190 |

##### 4.3.1.1.66	FDD reference test frequencies for operating band 66

Table 4.3.1.1.66-1: Test frequencies for E-UTRA channel bandwidth for operating band 66

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 1.4 | 131979 | 1710.7 | 66443 | 2110.7 |
|  | 3 | 131987 | 1711.5 | 66451 | 2111.5 |
|  | 5 | 131997 | 1712.5 | 66461 | 2112.5 |
|  | 10 | 132022 | 1715 | 66486 | 2115 |
|  | 15 | 132047 | 1717.5 | 66511 | 2117.5 |
|  | 20 | 132072 | 1720 | 66536 | 2120 |
| Mid Range Tx1 | 1.4/3/5/10/15/20 | 132322 | 1745 | 66786 | 2145 |
| Mid Range | 1.4/3/5/10/15/20 | 132422 | 1755 | 66886 | 2155 |
| Paired High Range2 | 1.4 | 132665 | 1779.3 | 67129 | 2179.3 |
|  | 3 | 132657 | 1778.5 | 67121 | 2178.5 |
|  | 5 | 132647 | 1777.5 | 67111 | 2177.5 |
|  | 10 | 132622 | 1775 | 67086 | 2175 |
|  | 15 | 132597 | 1772.5 | 67061 | 2172.5 |
|  | 20 | 132572 | 1770 | 67036 | 2170 |
| High Range3 | 1.4 | NA | NA | 67329 | 2199.3 |
|  | 3 | NA | NA | 67321 | 2198.5 |
|  | 5 | NA | NA | 67311 | 2197.5 |
|  | 10 | NA | NA | 67286 | 2195 |
|  | 15 | NA | NA | 67261 | 2192.5 |
|  | 20 | NA | NA | 67236 | 2190 |
| Note 1: Applicable for transmitter testing. Note 2: Applicable if UL is configured on the CC. Note 3: Applicable if no UL is configured on the CC. |  |  |  |  |  |

##### 4.3.1.1.66A	FDD reference test frequencies for CA in operating band 66

Table 4.3.1.1.66A-1: Test frequencies for CA_66B

| Range | CC-Combo / NRB_agg [RB] |  |  |  | CC1 Note1 |  |  |  |  |  | CC2 Note1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL |  | fUL [MHz] | NDL | fDL [MHz] | BW [RB] | NUL |  | fUL [MHz] | NDL | fDL [MHz] |
| Low | 25+25 | 25 | 131997 |  | 1712.5 | 66461 | 2112.5 | 25 | 132045 |  | 1717.3 | 66509 | 2117.3 |
|  | 25+50 | 25 | 132000 |  | 1712.8 | 66464 | 2112.8 | 50 | 132072 |  | 1720 | 66536 | 2120 |
|  |  | 50 | 132022 |  | 1715 | 66486 | 2115 | 25 | 132094 |  | 1722.2 | 66558 | 2122.2 |
|  | 25+75 | 25 | 132002 |  | 1713 | 66466 | 2113 | 75 | 132095 |  | 1722.3 | 66559 | 2122.3 |
|  |  | 75 | 132047 |  | 1717.5 | 66511 | 2117.5 | 25 | 132140 |  | 1726.8 | 66604 | 2126.8 |
|  | 50+50 | 50 | 132022 |  | 1715 | 66486 | 2115 | 50 | 132121 |  | 1724.9 | 66585 | 2124.9 |
| Mid | 25+25 | 25 | 132398 |  | 1752.6 | 66862 | 2152.6 | 25 | 132446 |  | 1757.4 | 66910 | 2157.4 |
|  | 25+50 | 25 | 132375 |  | 1750.3 | 66839 | 2150.3 | 50 | 132447 |  | 1757.5 | 66911 | 2157.5 |
|  |  | 50 | 132397 |  | 1752.5 | 66861 | 2152.5 | 25 | 132469 |  | 1759.7 | 66933 | 2159.7 |
|  | 25+75 | 25 | 132353 |  | 1748.1 | 66817 | 2148.1 | 75 | 132446 |  | 1757.4 | 66910 | 2157.4 |
|  |  | 75 | 132398 |  | 1752.6 | 66862 | 2152.6 | 25 | 132491 |  | 1761.9 | 66955 | 2161.9 |
|  | 50+50 | 50 | 132373 |  | 1750.1 | 66837 | 2150.1 | 50 | 132472 |  | 1760 | 66936 | 2160 |
| High2 | 25+25 | 25 | 132647 |  | 1777.5 | 67111 | 2177.5 | 25 | NA |  | NA | 67159 | 2182.3 |
|  | 25+50 | 25 | 132647 |  | 1777.5 | 67111 | 2177.5 | 50 | NA |  | NA | 67183 | 2184.7 |
|  |  | 50 | 132622 |  | 1775 | 67086 | 2175 | 25 | NA |  | NA | 67158 | 2182.2 |
|  | 25+75 | 25 | 132647 |  | 1777.5 | 67111 | 2177.5 | 75 | NA |  | NA | 67204 | 2186.8 |
|  |  | 75 | 132597 |  | 1772.5 | 67061 | 2172.5 | 25 | NA |  | NA | 67154 | 2181.8 |
|  | 50+50 | 50 | 132622 |  | 1775 | 67086 | 2175 | 50 | NA |  | NA | 67185 | 2184.9 |
| High3 | 25+25 | 25 | 132599 |  | 1772.7 | 67063 | 2172.7 | 25 | 132647 |  | 1777.5 | 67111 | 2177.5 |
|  | 25+50 | 25 | 132550 |  | 1767.8 | 67014 | 2167.8 | 50 | 132622 |  | 1775. | 67086 | 2175 |
|  |  | 50 | 132572 |  | 1770 | 67036 | 2170 | 25 | 132644 |  | 1777.2 | 67108 | 2177.2 |
|  | 25+75 | 25 | 132504 |  | 1763.2 | 66968 | 2163.2 | 75 | 132597 |  | 1772.5 | 67061 | 2172.5 |
|  |  | 75 | 132549 |  | 1767.7 | 67013 | 2167.7 | 25 | 132642 |  | 1777 | 67106 | 2177 |
|  | 50+50 | 50 | 132523 |  | 1765.1 | 66987 | 2165.1 | 50 | 132622 |  | 1775 | 67086 | 2175 |
| Note 1: Carriers in increasing frequency order. Note 2: Applicable for intra-band contiguous CA without UL CA. Note 3: Applicable for intra-band contiguous CA with UL CA. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.66A-2: Test frequencies for CA_66C

| Range | CC-Combo / NRB_agg [RB] |  |  | CC1 Note1 |  |  |  |  | CC2 Note1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL | fUL [MHz] | NDL | fDL [MHz] | BW [RB] | NUL | fUL [MHz] | NDL | fDL [MHz] |
| Low | 50+75 | 50 | 132025 | 1715.3 | 66489 | 2115.3 | 75 | 132145 | 1727.3 | 66609 | 2127.3 |
|  |  | 75 | 132047 | 1717.5 | 66511 | 2117.5 | 50 | 132167 | 1729.5 | 66631 | 2129.5 |
|  | 50+100 | 50 | 132027 | 1715.5 | 66491 | 2115.5 | 100 | 132171 | 1729.9 | 66635 | 2129.9 |
|  |  | 100 | 132072 | 1720 | 66536 | 2120 | 50 | 132216 | 1734.4 | 66680 | 2134.4 |
|  | 75+75 | 75 | 132047 | 1717.5 | 66511 | 2117.5 | 75 | 132197 | 1732.5 | 66661 | 2132.5 |
|  | 75+100 | 75 | 132050 | 1717.8 | 66514 | 2117.8 | 100 | 132221 | 1734.9 | 66685 | 2134.9 |
|  |  | 100 | 132072 | 1720 | 66536 | 2120 | 75 | 132243 | 1737.1 | 66707 | 2137.1 |
|  | 100+25 | 100 | 132072 | 1720 | 66536 | 2120 | 25 | 132189 | 1731.7 | 66653 | 2131.7 |
|  |  | 25 | 132005 | 1713.3 | 66469 | 2113.3 | 100 | 132122 | 1725.0 | 66586 | 2125.0 |
|  | 100+100 | 100 | 132072 | 1720 | 66536 | 2120 | 100 | 132270 | 1739.8 | 66734 | 2139.8 |
| Mid | 50+75 | 50 | 132351 | 1747.9 | 66815 | 2147.9 | 75 | 132471 | 1759.9 | 66935 | 2159.9 |
|  |  | 75 | 132373 | 1750.1 | 66837 | 2150.1 | 50 | 132493 | 1762.1 | 66957 | 2162.1 |
|  | 50+100 | 50 | 132328 | 1745.6 | 66792 | 2145.6 | 100 | 132472 | 1760 | 66936 | 2160 |
|  |  | 100 | 132373 | 1750.1 | 66837 | 2150.1 | 50 | 132517 | 1764.5 | 66981 | 2164.5 |
|  | 75+75 | 75 | 132347 | 1747.5 | 66811 | 2147.5 | 75 | 132497 | 1762.5 | 66961 | 2162.5 |
|  | 75+100 | 75 | 132325 | 1745.3 | 66789 | 2145.3 | 100 | 132496 | 1762.4 | 66960 | 2162.4 |
|  |  | 100 | 132348 | 1747.6 | 66812 | 2147.6 | 75 | 132519 | 1764.7 | 66983 | 2164.7 |
|  | 100+25 | 100 | 132397 | 1752.5 | 66861 | 2152.5 | 25 | 132514 | 1764.2 | 66978 | 2164.2 |
|  |  | 25 | 132330 | 1745.8 | 66794 | 2145.8 | 100 | 132447 | 1757.5 | 66911 | 2157.5 |
|  | 100+100 | 100 | 132323 | 1745.1 | 66787 | 2145.1 | 100 | 132521 | 1764.9 | 66985 | 2164.9 |
| High2 | 50+75 | 50 | 132622 | 1775 | 67086 | 2175 | 75 | NA | NA | 67206 | 2187 |
|  |  | 75 | 132597 | 1772.5 | 67061 | 2172.5 | 50 | NA | NA | 67181 | 2184.5 |
|  | 50+100 | 50 | 132622 | 1775 | 67086 | 2175 | 100 | NA | NA | 67230 | 2189.4 |
|  |  | 100 | 132572 | 1770 | 67036 | 2170 | 50 | NA | NA | 67180 | 2184.4 |
|  | 75+75 | 75 | 132597 | 1772.5 | 67061 | 2172.5 | 75 | NA | NA | 67211 | 2187.5 |
|  | 75+100 | 75 | 132597 | 1772.5 | 67061 | 2172.5 | 100 | NA | NA | 67232 | 2189.6 |
|  |  | 100 | 132572 | 1770 | 67036 | 2170 | 75 | NA | NA | 67207 | 2187.1 |
|  | 100+25 | 100 | 132572 | 1770 | 67036 | 2170 | 25 | NA | NA | 67153 | 2181.7 |
|  |  | 25 | 132647 | 1777.5 | 67111 | 2177.5 | 100 | NA | NA | 67228 | 2189.2 |
|  | 100+100 | 100 | 132572 | 1770 | 67036 | 2170 | 100 | NA | NA | 67234 | 2189.8 |
| High3 | 50+75 | 50 | 132477 | 1760.5 | 66941 | 2160.5 | 75 | 132597 | 1772.5 | 67061 | 2172.5 |
|  |  | 75 | 132499 | 1762.7 | 66963 | 2162.7 | 50 | 132619 | 1774.7 | 67083 | 2174.7 |
|  | 50+100 | 50 | 132428 | 1755.6 | 66892 | 2155.6 | 100 | 132572 | 1770 | 67036 | 2170 |
|  |  | 100 | 132473 | 1760.1 | 66937 | 2160.1 | 50 | 132617 | 1774.5 | 67081 | 2174.5 |
|  | 75+75 | 75 | 132447 | 1757.5 | 66911 | 2157.5 | 75 | 132597 | 1772.5 | 67061 | 2172.5 |
|  | 75+100 | 75 | 132401 | 1752.9 | 66885 | 2152.9 | 100 | 132572 | 1770 | 67036 | 2170 |
|  |  | 100 | 132423 | 1755.1 | 66887 | 2155.1 | 75 | 132594 | 1772.2 | 67058 | 2172.2 |
|  | 100+25 | 100 | 132522 | 1765 | 66986 | 2165 | 25 | 132639 | 1776.7 | 67103 | 2176.7 |
|  |  | 25 | 132455 | 1758.3 | 66919 | 2158.3 | 100 | 132572 | 1770.0 | 67036 | 2170.0 |
|  | 100+100 | 100 | 132374 | 1750.2 | 66838 | 2150.2 | 100 | 132572 | 1770 | 67036 | 2170 |
| Note 1: Carriers in increasing frequency order. Note 2: Applicable for intra-band contiguous CA without UL CA. Note 3: Applicable for intra-band contiguous CA with UL CA. |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.66A-3: Test frequencies for CA_66A-66A

| Test Frequency ID | CC-Combo / NRB_agg [RB] |  |  | CC1 Note1 |  |  | Wgap [MHz] |  |  | CC2 Note1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL | fUL [MHz] | NDL | fDL [MHz] |  | BW [RB] | NUL | fUL [MHz] | NDL | fDL [MHz] |
| Low WGap2 | 25+25 | 25 | 20125 | 1727.5 | 2125 | 2127.5 | 5 | 25 | NA | NA | 2225 | 2137.5 |
|  | 50+50 | 50 | 20100 | 1725 | 2100 | 2125 | 5 | 50 | NA | NA | 2250 | 2140 |
|  | 50+100 | 50 | 20095 | 1724.5 | 2095 | 2124.5 | 5 | 100 | NA | NA | 2255 | 2140.5 |
|  | 100+100 | 100 | 20050 | 1720 | 2050 | 2120 | 5 | 100 | NA | NA | 2300 | 2145 |
| Max WGap2 | 25+25 | 25 | 131997 | 1712.5 | 66461 | 2112.5 | 80 | 25 | NA | NA | 67311 | 2197.5 |
|  | 25+50 | 25 | 131997 | 1712.5 | 66461 | 2112.5 | 75 | 50 | NA | NA | 67286 | 2195 |
|  |  | 50 | 132022 | 1715 | 66486 | 2115 | 75 | 25 | NA | NA | 67311 | 2197.5 |
|  | 25+75 | 25 | 131997 | 1712.5 | 66461 | 2112.5 | 70 | 75 | NA | NA | 67261 | 2192.5 |
|  |  | 75 | 132047 | 1717.5 | 66511 | 2117.5 | 70 | 25 | NA | NA | 67311 | 2197.5 |
|  | 50+50 | 50 | 132022 | 1715 | 66486 | 2115 | 70 | 50 | NA | NA | 67286 | 2195 |
|  | 25+100 | 25 | 131997 | 1712.5 | 66461 | 2112.5 | 65 | 100 | NA | NA | 67236 | 2190 |
|  |  | 100 | 132072 | 1720 | 66536 | 2120 | 65 | 25 | NA | NA | 67311 | 2197.5 |
|  | 50+75 | 50 | 132022 | 1715 | 66486 | 2115 | 65 | 75 | NA | NA | 67261 | 2192.5 |
|  |  | 75 | 132047 | 1717.5 | 66511 | 2117.5 | 65 | 50 | NA | NA | 67286 | 2195 |
|  | 50+100 | 50 | 132022 | 1715 | 66486 | 2115 | 60 | 100 | NA | NA | 67236 | 2190 |
|  |  | 100 | 132072 | 1720 | 66536 | 2120 | 60 | 50 | NA | NA | 67286 | 2195 |
|  | 75+75 | 75 | 132047 | 1717.5 | 66511 | 2117.5 | 60 | 75 | NA | NA | 67261 | 2192.5 |
|  | 75+100 | 75 | 132047 | 1717.5 | 66511 | 2117.5 | 55 | 100 | NA | NA | 67236 | 2190 |
|  |  | 100 | 132072 | 1720 | 66536 | 2120 | 55 | 75 | NA | NA | 67261 | 2192.5 |
|  | 100+100 | 100 | 132072 | 1720 | 66536 | 2120 | 50 | 100 | NA | NA | 67236 | 2190 |
| Max WGap3 | 25+25 | 25 | 131997 | 1712.5 | 66461 | 2112.5 | 60 | 25 | 132647 | 1777.5 | 67111 | 2177.5 |
|  | 25+50 | 25 | 131997 | 1712.5 | 66461 | 2112.5 | 55 | 50 | 132622 | 1775 | 67086 | 2175 |
|  |  | 50 | 132022 | 1715 | 66486 | 2115 | 55 | 25 | 132647 | 1777.5 | 67111 | 2177.5 |
|  | 25+75 | 25 | 131997 | 1712.5 | 66461 | 2112.5 | 50 | 75 | 132597 | 1772.5 | 67061 | 2172.5 |
|  |  | 75 | 132047 | 1717.5 | 66511 | 2117.5 | 50 | 25 | 132647 | 1777.5 | 67111 | 2177.5 |
|  | 50+50 | 50 | 132022 | 1715 | 66486 | 2115 | 50 | 50 | 132622 | 1775 | 67086 | 2175 |
|  | 25+100 | 25 | 131997 | 1712.5 | 66461 | 2112.5 | 45 | 100 | 132572 | 1770 | 67036 | 2170 |
|  |  | 100 | 132072 | 1720 | 66536 | 2120 | 45 | 25 | 132647 | 1777.5 | 67111 | 2177.5 |
|  | 50+75 | 50 | 132022 | 1715 | 66486 | 2115 | 45 | 75 | 132597 | 1772.5 | 67061 | 2172.5 |
|  |  | 75 | 132047 | 1717.5 | 66511 | 2117.5 | 45 | 50 | 132622 | 1775 | 67086 | 2175 |
|  | 50+100 | 50 | 132022 | 1715 | 66486 | 2115 | 40 | 100 | 132572 | 1770 | 67036 | 2170 |
|  |  | 100 | 132072 | 1720 | 66536 | 2120 | 40 | 50 | 132622 | 1775 | 67086 | 2175 |
|  | 75+75 | 75 | 132047 | 1717.5 | 66511 | 2117.5 | 40 | 75 | 132597 | 1772.5 | 67061 | 2172.5 |
|  | 75+100 | 75 | 132047 | 1717.5 | 66511 | 2117.5 | 35 | 100 | 132572 | 1770 | 67036 | 2170 |
|  |  | 100 | 132072 | 1720 | 66536 | 2120 | 35 | 75 | 132597 | 1772.5 | 67061 | 2172.5 |
|  | 100+100 | 100 | 132072 | 1720 | 66536 | 2120 | 30 | 100 | 132572 | 1770 | 67036 | 2170 |
| Note 1: Carriers in increasing frequency order. Note 2: Applicable for intra-band non-contiguous CA without UL CA. Note 3: Applicable for intra-band non-contiguous CA with UL CA. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.66A-4: Test frequencies for CA_66D

| Range | CC-Combo / NRB_agg [RB] |  |  | CC1 Note1 |  |  |  |  | CC2 Note1 |  |  |  |  | CC3 Note1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL | fUL [MHz] | NDL | fDL [MHz] | BW [RB] | NUL | fUL [MHz] | NDL | fDL [MHz] | BW [RB] | NUL | fUL [MHz] | NDL | fDL [MHz] |
| Low | 25+100+100 | 25 | 132005 | 1713.3 | 66469 | 2113.3 | 100 | 132122 | 1725.0 | 66586 | 2125.0 | 100 | 132320 | 1744.8 | 66784 | 2144.8 |
|  |  | 100 | 132072 | 1720.0 | 66536 | 2120.0 | 25 | 132189 | 1731.7 | 66653 | 2131.7 | 100 | 132306 | 1743.4 | 66770 | 2143.4 |
|  |  | 100 | 132072 | 1720.0 | 66536 | 2120.0 | 100 | 132270 | 1739.8 | 66734 | 2139.8 | 25 | 132387 | 1751.5 | 66851 | 2151.5 |
|  | 50+75+100 | 50 | 132027 | 1715.5 | 66491 | 2115.5 | 75 | 132147 | 1727.5 | 66611 | 2127.5 | 100 | 132318 | 1744.6 | 66782 | 2144.6 |
|  |  | 50 | 132027 | 1715.5 | 66491 | 2115.5 | 100 | 132171 | 1729.9 | 66635 | 2129.9 | 75 | 132342 | 1747.0 | 66806 | 2147.0 |
|  |  | 75 | 132050 | 1717.8 | 66514 | 2117.8 | 50 | 132170 | 1729.8 | 66634 | 2129.8 | 100 | 132314 | 1744.2 | 66778 | 2144.2 |
|  |  | 75 | 132050 | 1717.8 | 66514 | 2117.8 | 100 | 132221 | 1734.9 | 66685 | 2134.9 | 50 | 132365 | 1749.3 | 66829 | 2149.3 |
|  |  | 100 | 132072 | 1720.0 | 66536 | 2120.0 | 50 | 132216 | 1734.4 | 66680 | 2134.4 | 75 | 132336 | 1746.4 | 66800 | 2146.4 |
|  |  | 100 | 132072 | 1720.0 | 66536 | 2120.0 | 75 | 132243 | 1737.1 | 66707 | 2137.1 | 50 | 132363 | 1749.1 | 66827 | 2149.1 |
|  | 75+75+75 | 75 | 132047 | 1717.5 | 66511 | 2117.5 | 75 | 132197 | 1732.5 | 66661 | 2132.5 | 75 | 132347 | 1747.5 | 66811 | 2147.5 |
|  | 75+75+100 | 75 | 132050 | 1717.8 | 66514 | 2117.8 | 75 | 132200 | 1732.8 | 66664 | 2132.8 | 100 | 132371 | 1749.9 | 66835 | 2149.9 |
|  |  | 75 | 132050 | 1717.8 | 66514 | 2117.8 | 100 | 132221 | 1734.9 | 66685 | 2134.9 | 75 | 132392 | 1752.0 | 66856 | 2152.0 |
|  |  | 100 | 132072 | 1720.0 | 66536 | 2120.0 | 75 | 132243 | 1737.1 | 66707 | 2137.1 | 75 | 132393 | 1752.1 | 66857 | 2152.1 |
|  | 50+100+100 | 50 | 132027 | 1715.5 | 66491 | 2115.5 | 100 | 132171 | 1729.9 | 66635 | 2129.9 | 100 | 132369 | 1749.7 | 66833 | 2149.7 |
|  |  | 100 | 132072 | 1720.0 | 66536 | 2120.0 | 50 | 132216 | 1734.4 | 66680 | 2134.4 | 100 | 132360 | 1748.8 | 66824 | 2148.8 |
|  |  | 100 | 132072 | 1720.0 | 66536 | 2120.0 | 100 | 132270 | 1739.8 | 66734 | 2139.8 | 50 | 132414 | 1754.2 | 66878 | 2154.2 |
|  | 75+100+100 | 75 | 132050 | 1717.8 | 66514 | 2117.8 | 100 | 132221 | 1734.9 | 66685 | 2134.9 | 100 | 132419 | 1754.7 | 66883 | 2154.7 |
|  |  | 100 | 132072 | 1720.0 | 66536 | 2120.0 | 75 | 132243 | 1737.1 | 66707 | 2137.1 | 100 | 132414 | 1754.2 | 66878 | 2154.2 |
|  |  | 100 | 132072 | 1720.0 | 66536 | 2120.0 | 100 | 132270 | 1739.8 | 66734 | 2139.8 | 75 | 132441 | 1756.9 | 66905 | 2156.9 |
|  | 100+100+100 | 100 | 132072 | 1720.0 | 66536 | 2120.0 | 100 | 132270 | 1739.8 | 66734 | 2139.8 | 100 | 132468 | 1759.6 | 66932 | 2159.6 |
| Mid | 25+100+100 | 25 | 132231 | 1735.9 | 66695 | 2135.9 | 100 | 132348 | 1747.6 | 66812 | 2147.6 | 100 | 132546 | 1767.4 | 67010 | 2167.4 |
|  |  | 100 | 132305 | 1743.3 | 66769 | 2143.3 | 25 | 132422 | 1755.0 | 66886 | 2155.0 | 100 | 132539 | 1766.7 | 67003 | 2166.7 |
|  |  | 100 | 132298 | 1742.6 | 66762 | 2142.6 | 100 | 132496 | 1762.4 | 66960 | 2162.4 | 25 | 132613 | 1774.1 | 67077 | 2174.1 |
|  | 50+75+100 | 50 | 132254 | 1738.2 | 66718 | 2138.2 | 75 | 132374 | 1750.2 | 66838 | 2150.2 | 100 | 132545 | 1767.3 | 67009 | 2167.3 |
|  |  | 50 | 132253 | 1738.1 | 66717 | 2138.1 | 100 | 132397 | 1752.5 | 66861 | 2152.5 | 75 | 132568 | 1769.6 | 67032 | 2169.6 |
|  |  | 75 | 132279 | 1740.7 | 66743 | 2140.7 | 50 | 132399 | 1752.7 | 66863 | 2152.7 | 100 | 132543 | 1767.1 | 67007 | 2167.1 |
|  |  | 75 | 132276 | 1740.4 | 66740 | 2140.4 | 100 | 132447 | 1757.5 | 66911 | 2157.5 | 50 | 132591 | 1771.9 | 67055 | 2171.9 |
|  |  | 100 | 132301 | 1742.9 | 66765 | 2142.9 | 50 | 132445 | 1757.3 | 66909 | 2157.3 | 75 | 132565 | 1769.3 | 67029 | 2169.3 |
|  |  | 100 | 132299 | 1742.7 | 66763 | 2142.7 | 75 | 132470 | 1759.8 | 66934 | 2159.8 | 50 | 132590 | 1771.8 | 67054 | 2171.8 |
|  | 75+75+75 | 75 | 132272 | 1740.0 | 66736 | 2140.0 | 75 | 132422 | 1755.0 | 66886 | 2155.0 | 75 | 132572 | 1770.0 | 67036 | 2170.0 |
|  | 75+75+100 | 75 | 132250 | 1737.8 | 66714 | 2137.8 | 75 | 132400 | 1752.8 | 66864 | 2152.8 | 100 | 132571 | 1769.9 | 67035 | 2169.9 |
|  |  | 75 | 132251 | 1737.9 | 66715 | 2137.9 | 100 | 132422 | 1755.0 | 66886 | 2155.0 | 75 | 132593 | 1772.1 | 67057 | 2172.1 |
|  |  | 100 | 132273 | 1740.1 | 66737 | 2140.1 | 75 | 132444 | 1757.2 | 66908 | 2157.2 | 75 | 132594 | 1772.2 | 67058 | 2172.2 |
|  | 50+100+100 | 50 | 132229 | 1735.7 | 66693 | 2135.7 | 100 | 132373 | 1750.1 | 66837 | 2150.1 | 100 | 132571 | 1769.9 | 67035 | 2169.9 |
|  |  | 100 | 132278 | 1740.6 | 66742 | 2140.6 | 50 | 132422 | 1755.0 | 66886 | 2155.0 | 100 | 132566 | 1769.4 | 67030 | 2169.4 |
|  |  | 100 | 132274 | 1740.2 | 66738 | 2140.2 | 100 | 132472 | 1760.0 | 66936 | 2160.0 | 50 | 132616 | 1774.4 | 67080 | 2174.4 |
|  | 75+100+100 | 75 | 132226 | 1735.4 | 66690 | 2135.4 | 100 | 132397 | 1752.5 | 66861 | 2152.5 | 100 | NA | NA | 67059 | 2172.3 |
|  |  | 100 | 132251 | 1737.9 | 66715 | 2137.9 | 75 | 132422 | 1755.0 | 66886 | 2155.0 | 100 | NA | NA | 67057 | 2172.1 |
|  |  | 100 | 132249 | 1737.7 | 66713 | 2137.7 | 100 | 132447 | 1757.5 | 66911 | 2157.5 | 75 | NA | NA | 67082 | 2174.6 |
|  | 100+100+100 | 100 | 132224 | 1735.2 | 66688 | 2135.2 | 100 | 132422 | 1755.0 | 66886 | 2155.0 | 100 | NA | NA | 67084 | 2174.8 |
| High | 25+100+100 | 25 | 132457 | 1758.5 | 66921 | 2158.5 | 100 | NA | NA | 67038 | 2170.2 | 100 | NA | NA | 67236 | 2190.0 |
|  |  | 100 | 132538 | 1766.6 | 67002 | 2166.6 | 25 | NA | NA | 67119 | 2178.3 | 100 | NA | NA | 67236 | 2190.0 |
|  |  | 100 | 132524 | 1765.2 | 66988 | 2165.2 | 100 | NA | NA | 67186 | 2185.0 | 25 | NA | NA | 67303 | 2196.7 |
|  | 50+75+100 | 50 | 132481 | 1760.9 | 66945 | 2160.9 | 75 | NA | NA | 67065 | 2172.9 | 100 | NA | NA | 67236 | 2190.0 |
|  |  | 50 | 132479 | 1760.7 | 66943 | 2160.7 | 100 | NA | NA | 67087 | 2175.1 | 75 | NA | NA | 67258 | 2192.2 |
|  |  | 75 | 132508 | 1763.6 | 66972 | 2163.6 | 50 | NA | NA | 67092 | 2175.6 | 100 | NA | NA | 67236 | 2190.0 |
|  |  | 75 | 132502 | 1763.0 | 66966 | 2163.0 | 100 | NA | NA | 67137 | 2180.1 | 50 | NA | NA | 67281 | 2194.5 |
|  |  | 100 | 132530 | 1765.8 | 66994 | 2165.8 | 50 | NA | NA | 67138 | 2180.2 | 75 | NA | NA | 67258 | 2192.2 |
|  |  | 100 | 132526 | 1765.4 | 66990 | 2165.4 | 75 | NA | NA | 67161 | 2182.5 | 50 | NA | NA | 67281 | 2194.5 |
|  | 75+75+75 | 75 | 132497 | 1762.5 | 66961 | 2162.5 | 75 | NA | NA | 67111 | 2177.5 | 75 | NA | NA | 67261 | 2192.5 |
|  | 75+75+100 | 75 | 132451 | 1757.9 | 66915 | 2157.9 | 75 | NA | NA | 67065 | 2172.9 | 100 | NA | NA | 67236 | 2190.0 |
|  |  | 75 | 132452 | 1758.0 | 66916 | 2158.0 | 100 | NA | NA | 67087 | 2175.1 | 75 | NA | NA | 67258 | 2192.2 |
|  |  | 100 | 132473 | 1760.1 | 66937 | 2160.1 | 75 | NA | NA | 67108 | 2177.2 | 75 | NA | NA | 67258 | 2192.2 |
|  | 50+100+100 | 50 | 132430 | 1755.8 | 66894 | 2155.8 | 100 | NA | NA | 67038 | 2170.2 | 100 | NA | NA | 67236 | 2190.0 |
|  |  | 100 | 132484 | 1761.2 | 66948 | 2161.2 | 50 | NA | NA | 67092 | 2175.6 | 100 | NA | NA | 67236 | 2190.0 |
|  |  | 100 | 132475 | 1760.3 | 66939 | 2160.3 | 100 | NA | NA | 67137 | 2180.1 | 50 | NA | NA | 67281 | 2194.5 |
|  | 75+100+100 | 75 | 132403 | 1753.1 | 66867 | 2153.1 | 100 | NA | NA | 67038 | 2170.2 | 100 | NA | NA | 67236 | 2190.0 |
|  |  | 100 | 132430 | 1755.8 | 66894 | 2155.8 | 75 | NA | NA | 67065 | 2172.9 | 100 | NA | NA | 67236 | 2190.0 |
|  |  | 100 | 132425 | 1755.3 | 66889 | 2155.3 | 100 | NA | NA | 67087 | 2175.1 | 75 | NA | NA | 67258 | 2192.2 |
|  | 100+100+100 | 100 | 132376 | 1750.4 | 66840 | 2150.4 | 100 | NA | NA | 67038 | 2170.2 | 100 | NA | NA | 67236 | 2190.0 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.66A-5: Test frequencies for CA_66A-66C

| Range | CC-Combo / |  |  | CC1 |  |  | Wgap [MHz] |  |  | CC2 |  |  |  |  | CC3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB_agg |  |  | Note1 |  |  |  |  |  | Note1 |  |  |  |  | Note1 |  |  |
|  | [RB] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BW | NUL | fUL | NDL | fDL |  | BW | NUL | fUL | NDL | fDL | BW | NUL | fUL | NDL | fDL |
|  |  | [RB] |  | [MHz] |  | [MHz] |  | [RB] |  | [MHz] |  | [MHz] | [RB] |  | [MHz] |  | [MHz] |
| Max Wgap | 25+25+100 | 25 | 131997 | 1712.5 | 66461 | 2112.5 | 60 | 100 | NA | NA | 67186 | 2185 | 25 | NA | NA | 67303 | 2196.7 |
|  |  | 25 | 131997 | 1712.5 | 66461 | 2112.5 | 60.8 | 25 | NA | NA | 67119 | 2178.3 | 100 | NA | NA | 67236 | 2190 |
|  | 25+50+75 | 25 | 131997 | 1712.5 | 66461 | 2112.5 | 60.5 | 50 | NA | NA | 67141 | 2180.5 | 75 | NA | NA | 67261 | 2192.5 |
|  |  | 25 | 131997 | 1712.5 | 66461 | 2112.5 | 60.2 | 75 | NA | NA | 67163 | 2182.7 | 50 | NA | NA | 67283 | 2194.7 |
|  | 25+50+100 | 25 | 131997 | 1712.5 | 66461 | 2112.5 | 55.6 | 50 | NA | NA | 67092 | 2175.6 | 100 | NA | NA | 67236 | 2190 |
|  |  | 25 | 131997 | 1712.5 | 66461 | 2112.5 | 55.1 | 100 | NA | NA | 67137 | 2180.1 | 50 | NA | NA | 67281 | 2194.5 |
|  |  | 50 | 132022 | 1715 | 66486 | 2115 | 55 | 100 | NA | NA | 67186 | 2185 | 25 | NA | NA | 67303 | 2196.7 |
|  |  | 50 | 132022 | 1715 | 66486 | 2115 | 55.8 | 25 | NA | NA | 67119 | 2178.3 | 100 | NA | NA | 67236 | 2190 |
|  | 25+75+75 | 25 | 131997 | 1712.5 | 66461 | 2112.5 | 55 | 75 | NA | NA | 67111 | 2177.5 | 75 | NA | NA | 67261 | 2192.5 |
|  | 25+75+100 | 25 | 131997 | 1712.5 | 66461 | 2112.5 | 50.4 | 75 | NA | NA | 67065 | 2172.9 | 100 | NA | NA | 67236 | 2190 |
|  |  | 25 | 131997 | 1712.5 | 66461 | 2112.5 | 50.1 | 100 | NA | NA | 67087 | 2175.1 | 75 | NA | NA | 67258 | 2192.2 |
|  |  | 75 | 132047 | 1717.5 | 66511 | 2117.5 | 50 | 100 | NA | NA | 67186 | 2185 | 25 | NA | NA | 67303 | 2196.7 |
|  |  | 75 | 132047 | 1717.5 | 66511 | 2117.5 | 50.8 | 25 | NA | NA | 67119 | 2178.3 | 100 | NA | NA | 67236 | 2190 |
|  | 25+100+100 | 25 | 131997 | 1712.5 | 66461 | 2112.5 | 45.2 | 100 | NA | NA | 67038 | 2170.2 | 100 | NA | NA | 67236 | 2190 |
|  | 25+100+100 | 100 | 132072 | 1720 | 66536 | 2120 | 45 | 100 | NA | NA | 67186 | 2185 | 25 | NA | NA | 67303 | 2196.7 |
|  |  | 100 | 132072 | 1720 | 66536 | 2120 | 45.8 | 25 | NA | NA | 67119 | 2178.3 | 100 | NA | NA | 67236 | 2190 |
|  | 50+50+75 | 50 | 132022 | 1715 | 66486 | 2115 | 55.5 | 50 | NA | NA | 67141 | 2180.5 | 75 | NA | NA | 67261 | 2192.5 |
|  |  | 50 | 132022 | 1715 | 66486 | 2115 | 55.2 | 75 | NA | NA | 67163 | 2182.7 | 50 | NA | NA | 67283 | 2194.7 |
|  | 50+50+100 | 50 | 132022 | 1715 | 66486 | 2115 | 50.6 | 50 | NA | NA | 67092 | 2175.6 | 100 | NA | NA | 67236 | 2190 |
|  |  | 50 | 132022 | 1715 | 66486 | 2115 | 50.1 | 100 | NA | NA | 67137 | 2180.1 | 50 | NA | NA | 67281 | 2194.5 |
|  | 50+75+75 | 50 | 132022 | 1715 | 66486 | 2115 | 50 | 75 | NA | NA | 67111 | 2177.5 | 75 | NA | NA | 67261 | 2192.5 |
|  |  | 75 | 132047 | 1717.5 | 66511 | 2117.5 | 50.5 | 50 | NA | NA | 67141 | 2180.5 | 75 | NA | NA | 67261 | 2192.5 |
|  |  | 75 | 132047 | 1717.5 | 66511 | 2117.5 | 50.2 | 75 | NA | NA | 67163 | 2182.7 | 50 | NA | NA | 67283 | 2194.7 |
|  | 50+75+100 | 50 | 132022 | 1715 | 66486 | 2115 | 45.4 | 75 | NA | NA | 67065 | 2172.9 | 100 | NA | NA | 67236 | 2190 |
|  |  | 50 | 132022 | 1715 | 66486 | 2115 | 45.1 | 100 | NA | NA | 67087 | 2175.1 | 75 | NA | NA | 67258 | 2192.2 |
|  |  | 75 | 132047 | 1717.5 | 66511 | 2117.5 | 45.6 | 50 | NA | NA | 67092 | 2175.6 | 100 | NA | NA | 67236 | 2190 |
|  |  | 100 | 132072 | 1720 | 66536 | 2120 | 45.5 | 50 | NA | NA | 67141 | 2180.5 | 75 | NA | NA | 67261 | 2192.5 |
|  |  | 100 | 132072 | 1720 | 66536 | 2120 | 45.2 | 75 | NA | NA | 67163 | 2182.7 | 50 | NA | NA | 67283 | 2194.7 |
|  |  | 75 | 132047 | 1717.5 | 66511 | 2117.5 | 45.1 | 100 | NA | NA | 67137 | 2180.1 | 50 | NA | NA | 67281 | 2194.5 |
|  |  | 100 | 132072 | 1720 | 66536 | 2120 | 45.5 | 50 | NA | NA | 67141 | 2180.5 | 75 | NA | NA | 67261 | 2192.5 |
|  |  | 100 | 132072 | 1720 | 66536 | 2120 | 45.2 | 75 | NA | NA | 67163 | 2182.7 | 50 | NA | NA | 67283 | 2194.7 |
|  | 50+100+100 | 50 | 132022 | 1715 | 66486 | 2115 | 40.2 | 100 | NA | NA | 67038 | 2170.2 | 100 | NA | NA | 67236 | 2190 |
|  |  | 100 | 132072 | 1720 | 66536 | 2120 | 40.6 | 50 | NA | NA | 67092 | 2175.6 | 100 | NA | NA | 67236 | 2190 |
|  |  | 100 | 132072 | 1720 | 66536 | 2120 | 40.1 | 100 | NA | NA | 67137 | 2180.1 | 50 | NA | NA | 67281 | 2194.5 |
|  | 75+75+75 | 75 | 132047 | 1717.5 | 66511 | 2117.5 | 45 | 75 | NA | NA | 67111 | 2177.5 | 75 | NA | NA | 67261 | 2192.5 |
|  | 75+75+100 | 75 | 132047 | 1717.5 | 66511 | 2117.5 | 40.4 | 75 | NA | NA | 67065 | 2172.9 | 100 | NA | NA | 67236 | 2190 |
|  |  | 75 | 132047 | 1717.5 | 66511 | 2117.5 | 40.1 | 100 | NA | NA | 67087 | 2175.1 | 75 | NA | NA | 67258 | 2192.2 |
|  |  | 100 | 132072 | 1720 | 66536 | 2120 | 40 | 75 | NA | NA | 67111 | 2177.5 | 75 | NA | NA | 67261 | 2192.5 |
|  | 75+100+100 | 75 | 132047 | 1717.5 | 66511 | 2117.5 | 35.2 | 100 | NA | NA | 67038 | 2170.2 | 100 | NA | NA | 67236 | 2190 |
|  |  | 100 | 132072 | 1720 | 66536 | 2120 | 35.4 | 75 | NA | NA | 67065 | 2172.9 | 100 | NA | NA | 67236 | 2190 |
|  |  | 100 | 132072 | 1720 | 66536 | 2120 | 35.1 | 100 | NA | NA | 67087 | 2175.1 | 75 | NA | NA | 67258 | 2192.2 |
|  | 100+100+100 | 100 | 132072 | 1720 | 66536 | 2120 | 30.2 | 100 | NA | NA | 67038 | 2170.2 | 100 | NA | NA | 67236 | 2190 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.66A-6: Test frequencies for CA_66C-66A

| Range | CC-Combo / |  |  | CC1 |  |  |  |  | CC2 |  |  | Wgap [MHz] |  |  | CC3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB_agg |  |  | Note1 |  |  |  |  | Note1 |  |  |  |  |  | Note1 |  |  |
|  | [RB] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BW | NUL | fUL | NDL | fDL | BW | NUL | fUL | NDL | fDL |  | BW | NUL | fUL | NDL | fDL |
|  |  | [RB] |  | [MHz] |  | [MHz] | [RB] |  | [MHz] |  | [MHz] |  | [RB] |  | [MHz] |  | [MHz] |
| Max Wgap | 25+25+100 | 100 | 132072 | 1720 | 66536 | 2120 | 25 | 132189 | 1731.7 | 66653 | 2131.7 | 60.8 | 25 | NA | NA | 67311 | 2197.5 |
|  |  | 25 | 132005 | 1713.3 | 66469 | 2113.3 | 100 | 132122 | 1725 | 66586 | 2125 | 60 | 25 | NA | NA | 67311 | 2197.5 |
|  | 25+50+75 | 50 | 132025 | 1715.3 | 66489 | 2115.3 | 75 | 132145 | 1727.3 | 66609 | 2127.3 | 60.2 | 25 | NA | NA | 67311 | 2197.5 |
|  |  | 75 | 132047 | 1717.5 | 66511 | 2117.5 | 50 | 132167 | 1729.5 | 66631 | 2129.5 | 60.5 | 25 | NA | NA | 67311 | 2197.5 |
|  | 25+50+100 | 50 | 132027 | 1715.5 | 66491 | 2115.5 | 100 | 132171 | 1729.9 | 66635 | 2129.9 | 55.1 | 25 | NA | NA | 67311 | 2197.5 |
|  |  | 100 | 132072 | 1720 | 66536 | 2120 | 50 | 132216 | 1734.4 | 66680 | 2134.4 | 55.6 | 25 | NA | NA | 67311 | 2197.5 |
|  |  | 100 | 132072 | 1720 | 66536 | 2120 | 25 | 132189 | 1731.7 | 66653 | 2131.7 | 55.8 | 50 | NA | NA | 67286 | 2195 |
|  |  | 25 | 132005 | 1713.3 | 66469 | 2113.3 | 100 | 132122 | 1725 | 66586 | 2125 | 55 | 50 | NA | NA | 67286 | 2195 |
|  | 25+75+75 | 75 | 132047 | 1717.5 | 66511 | 2117.5 | 75 | 132197 | 1732.5 | 66661 | 2132.5 | 55 | 25 | NA | NA | 67311 | 2197.5 |
|  | 25+75+100 | 75 | 132050 | 1717.8 | 66514 | 2117.8 | 100 | 132221 | 1734.9 | 66685 | 2134.9 | 50.1 | 25 | NA | NA | 67311 | 2197.5 |
|  |  | 100 | 132072 | 1720 | 66536 | 2120 | 75 | 132243 | 1737.1 | 66707 | 2137.1 | 50.4 | 25 | NA | NA | 67311 | 2197.5 |
|  |  | 100 | 132072 | 1720 | 66536 | 2120 | 25 | 132189 | 1731.7 | 66653 | 2131.7 | 50.8 | 75 | NA | NA | 67261 | 2192.5 |
|  |  | 25 | 132005 | 1713.3 | 66469 | 2113.3 | 100 | 132122 | 1725 | 66586 | 2125 | 50 | 75 | NA | NA | 67261 | 2192.5 |
|  | 25+100+100 | 100 | 132072 | 1720 | 66536 | 2120 | 100 | 132270 | 1739.8 | 66734 | 2139.8 | 45.2 | 25 | NA | NA | 67311 | 2197.5 |
|  | 25+100+100 | 100 | 132072 | 1720 | 66536 | 2120 | 25 | 132189 | 1731.7 | 66653 | 2131.7 | 45.8 | 100 | NA | NA | 67236 | 2190 |
|  |  | 25 | 132005 | 1713.3 | 66469 | 2113.3 | 100 | 132122 | 1725 | 66586 | 2125 | 45 | 100 | NA | NA | 67236 | 2190 |
|  | 50+50+75 | 50 | 132025 | 1715.3 | 66489 | 2115.3 | 75 | 132145 | 1727.3 | 66609 | 2127.3 | 55.2 | 50 | NA | NA | 67286 | 2195 |
|  |  | 75 | 132047 | 1717.5 | 66511 | 2117.5 | 50 | 132167 | 1729.5 | 66631 | 2129.5 | 55.5 | 50 | NA | NA | 67286 | 2195 |
|  | 50+50+100 | 50 | 132027 | 1715.5 | 66491 | 2115.5 | 100 | 132171 | 1729.9 | 66635 | 2129.9 | 50.1 | 50 | NA | NA | 67286 | 2195 |
|  |  | 100 | 132072 | 1720 | 66536 | 2120 | 50 | 132216 | 1734.4 | 66680 | 2134.4 | 50.6 | 50 | NA | NA | 67286 | 2195 |
|  | 50+75+75 | 75 | 132047 | 1717.5 | 66511 | 2117.5 | 75 | 132197 | 1732.5 | 66661 | 2132.5 | 50 | 50 | NA | NA | 67286 | 2195 |
|  |  | 50 | 132025 | 1715.3 | 66489 | 2115.3 | 75 | 132145 | 1727.3 | 66609 | 2127.3 | 50.2 | 75 | NA | NA | 67261 | 2192.5 |
|  |  | 75 | 132047 | 1717.5 | 66511 | 2117.5 | 50 | 132167 | 1729.5 | 66631 | 2129.5 | 50.5 | 75 | NA | NA | 67261 | 2192.5 |
|  | 50+75+100 | 75 | 132050 | 1717.8 | 66514 | 2117.8 | 100 | 132221 | 1734.9 | 66685 | 2134.9 | 45.1 | 50 | NA | NA | 67286 | 2195 |
|  |  | 100 | 132072 | 1720 | 66536 | 2120 | 75 | 132243 | 1737.1 | 66707 | 2137.1 | 45.4 | 50 | NA | NA | 67286 | 2195 |
|  |  | 50 | 132027 | 1715.5 | 66491 | 2115.5 | 100 | 132171 | 1729.9 | 66635 | 2129.9 | 45.1 | 75 | NA | NA | 67261 | 2192.5 |
|  |  | 50 | 132025 | 1715.3 | 66489 | 2115.3 | 75 | 132145 | 1727.3 | 66609 | 2127.3 | 45.2 | 100 | NA | NA | 67236 | 2190 |
|  |  | 75 | 132047 | 1717.5 | 66511 | 2117.5 | 50 | 132167 | 1729.5 | 66631 | 2129.5 | 45.5 | 100 | NA | NA | 67236 | 2190 |
|  |  | 100 | 132072 | 1720 | 66536 | 2120 | 50 | 132216 | 1734.4 | 66680 | 2134.4 | 45.6 | 75 | NA | NA | 67261 | 2192.5 |
|  | 50+100+100 | 100 | 132072 | 1720 | 66536 | 2120 | 100 | 132270 | 1739.8 | 66734 | 2139.8 | 40.2 | 50 | NA | NA | 67286 | 2195 |
|  |  | 50 | 132027 | 1715.5 | 66491 | 2115.5 | 100 | 132171 | 1729.9 | 66635 | 2129.9 | 40.1 | 100 | NA | NA | 67236 | 2190 |
|  |  | 100 | 132072 | 1720 | 66536 | 2120 | 50 | 132216 | 1734.4 | 66680 | 2134.4 | 40.6 | 100 | NA | NA | 67236 | 2190 |
|  | 75+75+75 | 75 | 132047 | 1717.5 | 66511 | 2117.5 | 75 | 132197 | 1732.5 | 66661 | 2132.5 | 45 | 75 | NA | NA | 67261 | 2192.5 |
|  | 75+75+100 | 75 | 132050 | 1717.8 | 66514 | 2117.8 | 100 | 132221 | 1734.9 | 66685 | 2134.9 | 40.1 | 75 | NA | NA | 67261 | 2192.5 |
|  |  | 100 | 132072 | 1720 | 66536 | 2120 | 75 | 132243 | 1737.1 | 66707 | 2137.1 | 40.4 | 75 | NA | NA | 67261 | 2192.5 |
|  |  | 75 | 132047 | 1717.5 | 66511 | 2117.5 | 75 | 132197 | 1732.5 | 66661 | 2132.5 | 40 | 100 | NA | NA | 67236 | 2190 |
|  | 75+100+100 | 100 | 132072 | 1720 | 66536 | 2120 | 100 | 132270 | 1739.8 | 66734 | 2139.8 | 35.2 | 75 | NA | NA | 67261 | 2192.5 |
|  |  | 75 | 132050 | 1717.8 | 66514 | 2117.8 | 100 | 132221 | 1734.9 | 66685 | 2134.9 | 35.1 | 100 | NA | NA | 67236 | 2190 |
|  |  | 100 | 132072 | 1720 | 66536 | 2120 | 75 | 132243 | 1737.1 | 66707 | 2137.1 | 35.4 | 100 | NA | NA | 67236 | 2190 |
|  | 100+100+100 | 100 | 132072 | 1720 | 66536 | 2120 | 100 | 132270 | 1739.8 | 66734 | 2139.8 | 30.2 | 100 | NA | NA | 67236 | 2190 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.66A-7: Test frequencies for CA_66A-66B

| Range | CC-Combo / |  |  | CC1 |  |  | Wgap [MHz] |  |  | CC2 |  |  |  |  | CC3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB_agg |  |  | Note1 |  |  |  |  |  | Note1 |  |  |  |  | Note1 |  |  |
|  | [RB] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BW | NUL | fUL | NDL | fDL |  | BW | NUL | fUL | NDL | fDL | BW | NUL | fUL | NDL | fDL |
|  |  | [RB] |  | [MHz] |  | [MHz] |  | [RB] |  | [MHz] |  | [MHz] | [RB] |  | [MHz] |  | [MHz] |
| Max Wgap | 25+25+25 | 25 | 131997 | 1712.5 | 66461 | 2112.5 | 75.2 | 25 | NA | NA | 67263 | 2192.7 | 25 | NA | NA | 67311 | 2197.5 |
|  | 25+25+50 | 25 | 131997 | 1712.5 | 66461 | 2112.5 | 70.3 | 25 | NA | NA | 67214 | 2187.8 | 50 | NA | NA | 67286 | 2195 |
|  |  | 25 | 131997 | 1712.5 | 66461 | 2112.5 | 70 | 50 | NA | NA | 67236 | 2190 | 25 | NA | NA | 67308 | 2197.2 |
|  | 25+25+50 | 50 | 132022 | 1715 | 66486 | 2115 | 70.2 | 25 | NA | NA | 67263 | 2192.7 | 25 | NA | NA | 67311 | 2197.5 |
|  | 25+25+75 | 25 | 131997 | 1712.5 | 66461 | 2112.5 | 65.7 | 25 | NA | NA | 67168 | 2183.2 | 75 | NA | NA | 67261 | 2192.5 |
|  |  | 25 | 131997 | 1712.5 | 66461 | 2112.5 | 65.2 | 75 | NA | NA | 67213 | 2187.7 | 25 | NA | NA | 67306 | 2197 |
|  | 25+25+75 | 75 | 132047 | 1717.5 | 66511 | 2117.5 | 65.2 | 25 | NA | NA | 67263 | 2192.7 | 25 | NA | NA | 67311 | 2197.5 |
|  | 25+25+100 | 100 | 132072 | 1720 | 66536 | 2120 | 60.2 | 25 | NA | NA | 67263 | 2192.7 | 25 | NA | NA | 67311 | 2197.5 |
|  | 25+50+50 | 25 | 131997 | 1712.5 | 66461 | 2112.5 | 65.1 | 50 | NA | NA | 67187 | 2185.1 | 50 | NA | NA | 67286 | 2195 |
|  |  | 50 | 132022 | 1715 | 66486 | 2115 | 65.3 | 25 | NA | NA | 67214 | 2187.8 | 50 | NA | NA | 67286 | 2195 |
|  |  | 50 | 132022 | 1715 | 66486 | 2115 | 65 | 50 | NA | NA | 67236 | 2190 | 25 | NA | NA | 67308 | 2197.2 |
|  | 25+50+75 | 50 | 132022 | 1715 | 66486 | 2115 | 60.7 | 25 | NA | NA | 67168 | 2183.2 | 75 | NA | NA | 67261 | 2192.5 |
|  |  | 50 | 132022 | 1715 | 66486 | 2115 | 60.2 | 75 | NA | NA | 67213 | 2187.7 | 25 | NA | NA | 67306 | 2197 |
|  | 25+50+75 | 75 | 132047 | 1717.5 | 66511 | 2117.5 | 60.3 | 25 | NA | NA | 67214 | 2187.8 | 50 | NA | NA | 67286 | 2195 |
|  |  | 75 | 132047 | 1717.5 | 66511 | 2117.5 | 60 | 50 | NA | NA | 67236 | 2190 | 25 | NA | NA | 67308 | 2197.2 |
|  | 25+50+100 | 100 | 132072 | 1720 | 66536 | 2120 | 55.3 | 25 | NA | NA | 67214 | 2187.8 | 50 | NA | NA | 67286 | 2195 |
|  |  | 100 | 132072 | 1720 | 66536 | 2120 | 55 | 50 | NA | NA | 67236 | 2190 | 25 | NA | NA | 67308 | 2197.2 |
|  | 25+75+75 | 75 | 132047 | 1717.5 | 66511 | 2117.5 | 55.7 | 25 | NA | NA | 67168 | 2183.2 | 75 | NA | NA | 67261 | 2192.5 |
|  |  | 75 | 132047 | 1717.5 | 66511 | 2117.5 | 55.2 | 75 | NA | NA | 67213 | 2187.7 | 25 | NA | NA | 67306 | 2197 |
|  | 25+75+100 | 100 | 132072 | 1720 | 66536 | 2120 | 50.7 | 25 | NA | NA | 67168 | 2183.2 | 75 | NA | NA | 67261 | 2192.5 |
|  |  | 100 | 132072 | 1720 | 66536 | 2120 | 50.2 | 75 | NA | NA | 67213 | 2187.7 | 25 | NA | NA | 67306 | 2197 |
|  | 50+50+50 | 50 | 132022 | 1715 | 66486 | 2115 | 60.1 | 50 | NA | NA | 67187 | 2185.1 | 50 | NA | NA | 67286 | 2195 |
|  | 50+50+75 | 75 | 132047 | 1717.5 | 66511 | 2117.5 | 55.1 | 50 | NA | NA | 67187 | 2185.1 | 50 | NA | NA | 67286 | 2195 |
|  | 50+50+100 | 100 | 132072 | 1720 | 66536 | 2120 | 50.1 | 50 | NA | NA | 67187 | 2185.1 | 50 | NA | NA | 67286 | 2195 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.66A-8: Test frequencies for CA_66B-66A

| Range | CC-Combo / |  |  | CC1 |  |  |  |  | CC2 |  |  | Wgap [MHz] |  |  | CC3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB_agg |  |  | Note1 |  |  |  |  | Note1 |  |  |  |  |  | Note1 |  |  |
|  | [RB] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BW | NUL | fUL | NDL | fDL | BW | NUL | fUL | NDL | fDL |  | BW | NUL | fUL | NDL | fDL |
|  |  | [RB] |  | [MHz] |  | [MHz] | [RB] |  | [MHz] |  | [MHz] |  | [RB] |  | [MHz] |  | [MHz] |
| Max Wgap | 25+25+25 | 25 | 131997 | 1712.5 | 66461 | 2112.5 | 25 | 132045 | 1717.3 | 66509 | 2117.3 | 75.2 | 25 | NA | NA | 67311 | 2197.5 |
|  | 25+25+50 | 25 | 132000 | 1712.8 | 66464 | 2112.8 | 50 | 132072 | 1720 | 66536 | 2120 | 70 | 25 | NA | NA | 67311 | 2197.5 |
|  |  | 50 | 132022 | 1715 | 66486 | 2115 | 25 | 132094 | 1722.2 | 66558 | 2122.2 | 70.3 | 25 | NA | NA | 67311 | 2197.5 |
|  | 25+25+50 | 25 | 131997 | 1712.5 | 66461 | 2112.5 | 25 | 132045 | 1717.3 | 66509 | 2117.3 | 70.2 | 50 | NA | NA | 67286 | 2195 |
|  | 25+25+75 | 25 | 132002 | 1713 | 66466 | 2113 | 75 | 132095 | 1722.3 | 66559 | 2122.3 | 65.2 | 25 | NA | NA | 67311 | 2197.5 |
|  |  | 75 | 132047 | 1717.5 | 66511 | 2117.5 | 25 | 132140 | 1726.8 | 66604 | 2126.8 | 65.7 | 25 | NA | NA | 67311 | 2197.5 |
|  | 25+25+75 | 25 | 131997 | 1712.5 | 66461 | 2112.5 | 25 | 132045 | 1717.3 | 66509 | 2117.3 | 65.2 | 75 | NA | NA | 67261 | 2192.5 |
|  | 25+25+100 | 25 | 131997 | 1712.5 | 66461 | 2112.5 | 25 | 132045 | 1717.3 | 66509 | 2117.3 | 60.2 | 100 | NA | NA | 67236 | 2190 |
|  | 25+50+50 | 50 | 132022 | 1715 | 66486 | 2115 | 50 | 132121 | 1724.9 | 66585 | 2124.9 | 65.1 | 25 | NA | NA | 67311 | 2197.5 |
|  |  | 25 | 132000 | 1712.8 | 66464 | 2112.8 | 50 | 132072 | 1720 | 66536 | 2120 | 65 | 50 | NA | NA | 67286 | 2195 |
|  |  | 50 | 132022 | 1715 | 66486 | 2115 | 25 | 132094 | 1722.2 | 66558 | 2122.2 | 65.3 | 50 | NA | NA | 67286 | 2195 |
|  | 25+50+75 | 25 | 132002 | 1713 | 66466 | 2113 | 75 | 132095 | 1722.3 | 66559 | 2122.3 | 60.2 | 50 | NA | NA | 67286 | 2195 |
|  |  | 75 | 132047 | 1717.5 | 66511 | 2117.5 | 25 | 132140 | 1726.8 | 66604 | 2126.8 | 60.7 | 50 | NA | NA | 67286 | 2195 |
|  | 25+50+75 | 25 | 132000 | 1712.8 | 66464 | 2112.8 | 50 | 132072 | 1720 | 66536 | 2120 | 60 | 75 | NA | NA | 67261 | 2192.5 |
|  |  | 50 | 132022 | 1715 | 66486 | 2115 | 25 | 132094 | 1722.2 | 66558 | 2122.2 | 60.3 | 75 | NA | NA | 67261 | 2192.5 |
|  | 25+50+100 | 25 | 132000 | 1712.8 | 66464 | 2112.8 | 50 | 132072 | 1720 | 66536 | 2120 | 55 | 100 | NA | NA | 67236 | 2190 |
|  |  | 50 | 132022 | 1715 | 66486 | 2115 | 25 | 132094 | 1722.2 | 66558 | 2122.2 | 55.3 | 100 | NA | NA | 67236 | 2190 |
|  | 25+75+75 | 25 | 132002 | 1713 | 66466 | 2113 | 75 | 132095 | 1722.3 | 66559 | 2122.3 | 55.2 | 75 | NA | NA | 67261 | 2192.5 |
|  |  | 75 | 132047 | 1717.5 | 66511 | 2117.5 | 25 | 132140 | 1726.8 | 66604 | 2126.8 | 55.7 | 75 | NA | NA | 67261 | 2192.5 |
|  | 25+75+100 | 25 | 132002 | 1713 | 66466 | 2113 | 75 | 132095 | 1722.3 | 66559 | 2122.3 | 50.2 | 100 | NA | NA | 67236 | 2190 |
|  |  | 75 | 132047 | 1717.5 | 66511 | 2117.5 | 25 | 132140 | 1726.8 | 66604 | 2126.8 | 50.7 | 100 | NA | NA | 67236 | 2190 |
|  | 50+50+50 | 50 | 132022 | 1715 | 66486 | 2115 | 50 | 132121 | 1724.9 | 66585 | 2124.9 | 60.1 | 50 | NA | NA | 67286 | 2195 |
|  | 50+50+75 | 50 | 132022 | 1715 | 66486 | 2115 | 50 | 132121 | 1724.9 | 66585 | 2124.9 | 55.1 | 75 | NA | NA | 67261 | 2192.5 |
|  | 50+50+100 | 50 | 132022 | 1715 | 66486 | 2115 | 50 | 132121 | 1724.9 | 66585 | 2124.9 | 50.1 | 100 | NA | NA | 67236 | 2190 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### 4.3.1.1.67	FDD reference test frequencies for CA in operating band 67

Table 4.3.1.1.67-1: Test frequencies for E-UTRA channel bandwidth for operating band 67

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 5 | NA[1] | NA | 67361 | 740.5 |
|  | 10 | NA[1] | NA | 67386 | 743 |
|  | 15 | NA[1] | NA | 67411 | 745.5 |
| Mid Range | 5/10/15/20 | NA[1] | NA | 67436 | 748 |
| High Range | 5 | NA[1] | NA | 67511 | 755.5 |
|  | 10 | NA[1] | NA | 67486 | 753 |
|  | 15 | NA[1] | NA | 67461 | 750.5 |
| NOTE 1: Restricted to E-UTRA operation when carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. |  |  |  |  |  |

##### 4.3.1.1.68	FDD reference test frequencies for operating band 68

Table 4.3.1.1.68-1: Test frequencies for E-UTRA channel bandwidth for operating band 68

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 5 | 132697 | 700.5 | 67561 | 755.5 |
|  | 10 | 132722 | 703 | 67586 | 758 |
|  | 15 | 132747 | 705.5 | 67611 | 760.5 |
| Mid Range | 5/10/15 | 132822 | 713 | 67686 | 768 |
| High Range | 5 | 132947 | 725.5 | 67811 | 780.5 |
|  | 10 | 132922 | 723 | 67786 | 778 |
|  | 15 | 132897 | 720.5 | 67761 | 775.5 |

##### 4.3.1.1.69	FDD reference test frequencies for operating band 69

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 5 | NA[1] | NA | 67861 | 2572.5 |
|  | 10 | NA[1] | NA | 67886 | 2575 |
|  | 15 | NA[1] | NA | 67911 | 2577.5 |
|  | 20 | NA[1] | NA | 67936 | 2580 |
| Mid Range | 5/10/15/20 | NA[1] | NA | 68086 | 2595 |
| High Range | 5 | NA[1] | NA | 68311 | 2617.5 |
|  | 10 | NA[1] | NA | 68286 | 2615 |
|  | 15 | NA[1] | NA | 68261 | 2612.5 |
|  | 20 | NA[1] | NA | 68236 | 2610 |
| NOTE 1: Restricted to E-UTRA operation when carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. |  |  |  |  |  |

##### 4.3.1.1.70	FDD reference test frequencies for operating band 70

Table 4.3.1.1.70-1: Test frequencies for E-UTRA channel bandwidth for operating band 70

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 5 | 132997 | 1697.5 | 68361 | 1997.5 |
|  | 10 | 133022 | 1700 | 68386 | 2000 |
|  | 15 | 133047 | 1702.5 | 68411 | 2002.5 |
| Paired Mid Range1 | 5/10/15 | 133047 | 1702.5 | 68411 | 2002.5 |
| Mid Range2 | 5/10/15 | NA | NA | 68461 | 2007.5 |
| Paired High Range1 | 5 | 133097 | 1707.5 | 68461 | 2007.5 |
|  | 10 | 133072 | 1705 | 68436 | 2005 |
|  | 15 | 133047 | 1702.5 | 68411 | 2002.5 |
| High Range2 | 5 | NA | NA | 68561 | 2017.5 |
|  | 10 | NA | NA | 68536 | 2015 |
|  | 15 | NA | NA | 68511 | 2012.5 |
| Note 1: Applicable if UL is configured on the CC. Note 2: Applicable if no UL is configured on the CC. |  |  |  |  |  |

##### 4.3.1.1.70A	FDD reference test frequencies for CA in operating band 70

Table 4.3.1.1.70A-1: Test frequencies for CA_70C

| Test Fre-quency ID2 | CC-Combo / NRB_agg [RB] |  |  |  | CC1 Note1 |  |  |  |  |  | CC2 Note1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL |  | fUL [MHz] | NDL | fDL [MHz] | BW [RB] | NUL |  | fUL [MHz] | NDL | fDL [MHz] |
| 1 | 25+100 | 25 | 132997 |  | 1697.5 | 68361 | 1997.5 | 100 | NA |  | NA | 68478 | 2009.2 |
| 23 | 50+75 | 50 | 133072 |  | 1705 | 68386 | 2000 | 75 | NA |  | NA | 68506 | 2012 |
| 3 | 75+50 | 75 | 133047 |  | 1702.5 | 68411 | 2002.5 | 50 | NA |  | NA | 68531 | 2014.5 |
| Note 1: Carriers in increasing frequency order. Note 2: These reflect the CA operation scenarios defined for CA_70C. Note 3: This combination is for 295MHz Tx-Rx separation in CA operation. |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### 4.3.1.1.71	FDD reference test frequencies for operating band 71

Table 4.3.1.1.71-1: Test frequencies for E-UTRA channel bandwidth for operating band 71

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 5 | 133147 | 665.5 | 68611 | 619.5 |
|  | 10 | 133172 | 668 | 68636 | 622 |
|  | 15 | 133197 | 670.5 | 68661 | 624.5 |
|  | 20 | 133222 | 673 | 68686 | 627 |
| Mid Range | 5/10/15 | 133297 | 680.5 | 68761 | 634.5 |
|  | 20 | 133322 | 683 | 68786 | 637 |
| High Range | 5 | 133447 | 695.5 | 68911 | 649.5 |
|  | 10 | 133422 | 693 | 68886 | 647 |
|  | 15 | 133397 | 690.5 | 68861 | 644.5 |
|  | 20 | 133372 | 688 | 68836 | 642 |

##### 4.3.1.1.72	FDD reference test frequencies for operating band 72

Table 4.3.1.1.72-1: Test frequencies for E-UTRA channel bandwidth for operating band 72

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 1.4 | 133479 | 451.7 | 68943 | 461.7 |
|  | 3[1] | 133487 | 452.5 | 68951 | 462.5 |
|  | 5[1] | 133497 | 453.5 | 68961 | 463.5 |
| Mid Range | 1.4/3[1]/ 5[1] | 133497 | 453.5 | 68961 | 463.5 |
| High Range | 1.4 | 133515 | 455.3 | 68979 | 465.3 |
|  | 3[1] | 133507 | 454.5 | 68971 | 464.5 |
|  | 5[1] | 133497 | 453.5 | 68961 | 463.5 |
| NOTE 1: Bandwidth for which a relaxation of the specified UE receiver sensitivity requirement (TS 36.101 [27] Clause 7.3) is allowed. |  |  |  |  |  |

##### 4.3.1.1.73	FDD reference test frequencies for operating band 73

Table 4.3.1.1.73-1: Test frequencies for E-UTRA channel bandwidth for operating band 73

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 1.4 | 133529 | 450.7 | 68993 | 460.7 |
|  | 3[1] | 133537 | 451.5 | 69001 | 461.5 |
|  | 5[1] | 133547 | 452.5 | 69011 | 462.5 |
| Mid Range | 1.4/3[1]/ 5[1] | 133547 | 452.5 | 69011 | 462.5 |
| High Range | 1.4 | 133565 | 454.3 | 69029 | 464.3 |
|  | 3[1] | 133557 | 453.5 | 69021 | 463.5 |
|  | 5[1] | 133547 | 452.5 | 69011 | 462.5 |
| NOTE 1: Bandwidth for which a relaxation of the specified UE receiver sensitivity requirement (TS 36.101 [27] Clause 7.3) is allowed. |  |  |  |  |  |

##### 4.3.1.1.74	FDD reference test frequencies for operating band 74

Table 4.3.1.1.74-1: Test frequencies for E-UTRA channel bandwidth for operating band 74

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 1.4 | 133579 | 1427.7 | 69043 | 1475.7 |
|  | 3 | 133587 | 1428.5 | 69051 | 1476.5 |
|  | 5 | 133597 | 1429.5 | 69061 | 1477.5 |
|  | 10 | 133622 | 1432 | 69086 | 1480 |
|  | 15 | 133647 | 1434.5 | 69111 | 1482.5 |
|  | 20 | 133672 | 1437 | 69136 | 1485 |
| Mid Range | 1.4/3/5/10/15/20 | 133787 | 1448.5 | 69251 | 1496.5 |
| High Range | 1.4 | 133995 | 1469.3 | 69459 | 1517.3 |
|  | 3 | 133987 | 1468.5 | 69451 | 1516.5 |
|  | 5 | 133977 | 1467.5 | 69441 | 1515.5 |
|  | 10 | 133952 | 1465 | 69416 | 1513 |
|  | 15 | 133927 | 1434.5 | 69391 | 1510.5 |
|  | 20 | 133902 | 1460 | 69366 | 1508 |

##### 4.3.1.1.75 to 4.3.1.1.84	FFS

Note:	Sub-clauses 4.3.1.1.75 to 4.3.1.1.84 are marked as FFS as bands 75 to 84 are not yet defined in TS 36.101 [27].

##### 4.3.1.1.85	FDD reference test frequencies for operating band 85

Table 4.3.1.1.85-1: Test frequencies for E-UTRA channel bandwidth for operating band 85

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 5 [1] | 134027 | 700.5 | 70391 | 730.5 |
|  | 10 [1] | 134052 | 703 | 70416 | 733 |
| Mid Range | 5 [1]/10 [1] | 134092 | 707 | 70456 | 737 |
| High Range | 5 [1] | 134157 | 713.5 | 70521 | 743.5 |
|  | 10 [1] | 134132 | 711 | 70496 | 741 |
| NOTE 1: Bandwidth for which a relaxation of the specified UE receiver sensitivity requirement (TS 36.101 [27] Clause 7.3) is allowed. |  |  |  |  |  |

##### 4.3.1.1.86	FFS

Note:	Sub-clause 4.3.1.1.86 is marked as FFS as band 86 is not yet defined in TS 36.101 [27].

##### 4.3.1.1.87	FDD reference test frequencies for operating band 87

Table 4.3.1.1.87-1: Test frequencies for E-UTRA channel bandwidth for operating band 87

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 1.4 | 134189 | 410.7 | 70553 | 420.7 |
|  | 3[1] | 134197 | 411.5 | 70561 | 421.5 |
|  | 5[1] | 134207 | 412.5 | 70571 | 422.5 |
| Mid Range | 1.4/3[1]/ 5[1] | 134207 | 412.5 | 70571 | 422.5 |
| High Range | 1.4 | 134225 | 414.3 | 70589 | 424.3 |
|  | 3[1] | 134217 | 413.5 | 70581 | 423.5 |
|  | 5[1] | 134207 | 412.5 | 70571 | 422.5 |
| NOTE 1: Bandwidth for which a relaxation of the specified UE receiver sensitivity requirement (TS 36.101 [27] Clause 7.3) is allowed. |  |  |  |  |  |

##### 4.3.1.1.88	FDD reference test frequencies for operating band 88

Table 4.3.1.1.88-1: Test frequencies for E-UTRA channel bandwidth for operating band 88

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 1.4 | 134239 | 412.7 | 70603 | 422.7 |
|  | 3[1] | 134247 | 413.5 | 70611 | 423.5 |
|  | 5[1] | 134257 | 414.5 | 70621 | 424.5 |
| Mid Range | 1.4/3[1]/ 5[1] | 134257 | 414.5 | 70621 | 424.5 |
| High Range | 1.4 | 134275 | 416.3 | 70639 | 426.3 |
|  | 3[1] | 134267 | 415.5 | 70631 | 425.5 |
|  | 5[1] | 134257 | 414.5 | 70621 | 424.5 |
| NOTE 1: Bandwidth for which a relaxation of the specified UE receiver sensitivity requirement (TS 36.101 [27] Clause 7.3) is allowed. |  |  |  |  |  |

#### 4.3.1.2	TDD Mode Test frequencies

##### 4.3.1.2.1	TDD reference test frequencies for Operating Band 33

Table 4.3.1.2.1-1: Test frequencies for E-UTRA channel bandwidth for operating band 33

| Test Frequency ID | Bandwidth [MHz] | EARFCN | Frequency (UL and DL) [MHz] |
| --- | --- | --- | --- |
| Low Range | 5 | 36025 | 1902.5 |
|  | 10 | 36050 | 1905 |
|  | 15 | 36075 | 1907.5 |
|  | 20 | 36100 | 1910 |
| Mid Range | 5/10/15/20 | 36100 | 1910 |
| High Range | 5 | 36175 | 1917.5 |
|  | 10 | 36150 | 1915 |
|  | 15 | 36125 | 1912.5 |
|  | 20 | 36100 | 1910 |

##### 4.3.1.2.2	TDD reference test frequencies for Operating Band 34

Table 4.3.1.2.2-1: Test frequencies for E-UTRA channel bandwidth for operating band 34

| Test Frequency ID | Bandwidth [MHz] | EARFCN | Frequency (UL and DL) [MHz] |
| --- | --- | --- | --- |
| Low Range | 5 | 36225 | 2012.5 |
|  | 10 | 36250 | 2015 |
|  | 15 | 36275 | 2017.5 |
| Mid Range | 5/10/15 | 36275 | 2017.5 |
| High Range | 5 | 36325 | 2022.5 |
|  | 10 | 36300 | 2020 |
|  | 15 | 36275 | 2017.5 |

##### 4.3.1.2.3	TDD reference test frequencies for Operating Band 35

Table 4.3.1.2.3-1: Test frequencies for E-UTRA channel bandwidth for operating band 35

| Test Frequency ID | Bandwidth [MHz] | EARFCN [MHz] | Frequency (UL and DL) [MHz] |
| --- | --- | --- | --- |
| Low Range | 1.4 | 36357 | 1850.7 |
|  | 3 | 36365 | 1851.5 |
|  | 5 | 36375 | 1852.5 |
|  | 10 | 36400 | 1855 |
|  | 15 | 36425 | 1857.5 |
|  | 20 | 36450 | 1860 |
| Mid Range | 1.4/3/5/10/15/20 | 36650 | 1880 |
| High Range | 1.4 | 36943 | 1909.3 |
|  | 3 | 36935 | 1908.5 |
|  | 5 | 36925 | 1907.5 |
|  | 10 | 36900 | 1905 |
|  | 15 | 36875 | 1902.5 |
|  | 20 | 36850 | 1900 |

##### 4.3.1.2.4	TDD reference test frequencies for Operating Band 36

Table 4.3.1.2.4-1: Test frequencies for E-UTRA channel bandwidth for operating band 36

| Test Frequency ID | Bandwidth [MHz] | EARFCN [MHz] | Frequency (UL and DL) [MHz] |
| --- | --- | --- | --- |
| Low Range | 1.4 | 36957 | 1930.7 |
|  | 3 | 36965 | 1931.5 |
|  | 5 | 36975 | 1932.5 |
|  | 10 | 37000 | 1935 |
|  | 15 | 37025 | 1937.5 |
|  | 20 | 37050 | 1940 |
| Mid Range | 1.4/3/5/10/15/20 | 37250 | 1960 |
| High Range | 1.4 | 37543 | 1989.3 |
|  | 3 | 37535 | 1988.5 |
|  | 5 | 37525 | 1987.5 |
|  | 10 | 37500 | 1985 |
|  | 15 | 37475 | 1982.5 |
|  | 20 | 37450 | 1980 |

##### 4.3.1.2.5	TDD reference test frequencies for Operating Band 37

Table 4.3.1.2.5-1: Test frequencies for E-UTRA channel bandwidth for operating band 37

| Test Frequency ID | Bandwidth [MHz] | EARFCN [MHz] | Frequency (UL and DL) [MHz] |
| --- | --- | --- | --- |
| Low Range | 5 | 37575 | 1912.5 |
|  | 10 | 37600 | 1915 |
|  | 15 | 37625 | 1917.5 |
|  | 20 | 37650 | 1920 |
| Mid Range | 5/10/15/20 | 37650 | 1920 |
| High Range | 5 | 37725 | 1927.5 |
|  | 10 | 37700 | 1925 |
|  | 15 | 37675 | 1922.5 |
|  | 20 | 37650 | 1920 |

##### 4.3.1.2.6	TDD reference test frequencies for Operating Band 38

Table 4.3.1.2.6-1: Test frequencies for E-UTRA channel bandwidth for operating band 38

| Test Frequency ID | Bandwidth [MHz] | EARFCN | Frequency (UL and DL) [MHz] |
| --- | --- | --- | --- |
| Low Range | 5 | 37775 | 2572.5 |
|  | 10 | 37800 | 2575 |
|  | 15 | 37825 | 2577.5 |
|  | 20 | 37850 | 2580 |
| Mid Range | 5/10/15/20 | 38000 | 2595 |
| High Range | 5 | 38225 | 2617.5 |
|  | 10 | 38200 | 2615 |
|  | 15 | 38175 | 2612.5 |
|  | 20 | 38150 | 2610 |

##### 4.3.1.2.6A	TDD reference test frequencies for CA in operating band 38

Table 4.3.1.2.6A-1: Test frequencies for CA_38C

| Range | CC-Combo / NRB_agg [RB] |  | CC1 Note1 |  |  | CC2 Note1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL/DL | fUL/DL [MHz] | BW [RB] | NUL/DL | fUL/DL [MHz] |
| Low | 75+75 | 75 | 37825 | 2577.5 | 75 | 37975 | 2592.5 |
|  | 100+100 | 100 | 37850 | 2580 | 100 | 38048 | 2599.8 |
| Mid | 75+75 | 75 | 37925 | 2587.5 | 75 | 38075 | 2602.5 |
|  | 100+100 | 100 | 37901 | 2585.1 | 100 | 38099 | 2604.9 |
| High | 75+75 | 75 | 38025 | 2597.5 | 75 | 38175 | 2612.5 |
|  | 100+100 | 100 | 37952 | 2590.2 | 100 | 38150 | 2610 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |

##### 4.3.1.2.7	TDD reference test frequencies for Operating Band 39

Table 4.3.1.2.7-1: Test frequencies for E-UTRA channel bandwidth for operating band 39

| Test Frequency ID | Bandwidth [MHz] | EARFCN | Frequency (UL and DL) [MHz] |
| --- | --- | --- | --- |
| Low Range | 5 | 38275 | 1882.5 |
|  | 10 | 38300 | 1885 |
|  | 15 | 38325 | 1887.5 |
|  | 20 | 38350 | 1890 |
| Mid Range | 5/10/15/20 | 38450 | 1900 |
| High Range | 5 | 38625 | 1917.5 |
|  | 10 | 38600 | 1915 |
|  | 15 | 38575 | 1912.5 |
|  | 20 | 38550 | 1910 |

##### 4.3.1.2.7A	TDD reference test frequencies for CA in Operating Band 39

Table 4.3.1.2.7A-1: Test frequencies for CA_39C

| Range | CC-Combo / NRB_agg [RB] |  | CC1 Note1 |  |  | CC2 Note1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL/DL | fUL/DL [MHz] | BW [RB] | NUL/DL | fUL/DL [MHz] |
| Low | 25+100 | 25 | 38283 | 1883.3 | 100 | 38400 | 1895.0 |
|  |  | 100 | 38350 | 1890 | 25 | 38467 | 1901.7 |
|  | 50+100 | 50 | 38305 | 1885.5 | 100 | 38449 | 1899.9 |
|  |  | 100 | 38350 | 1890 | 50 | 38494 | 1904.4 |
|  | 75+100 | 75 | 38328 | 1887.8 | 100 | 38499 | 1904.9 |
|  |  | 100 | 38350 | 1890 | 75 | 38521 | 1907.1 |
| Mid | 25+100 | 25 | 38358 | 1890.8 | 100 | 38475 | 1902.5 |
|  |  | 100 | 38425 | 1897.5 | 25 | 38542 | 1909.2 |
|  | 50+100 | 50 | 38356 | 1890.6 | 100 | 38500 | 1905.0 |
|  |  | 100 | 38401 | 1895.1 | 50 | 38545 | 1909.5 |
|  | 75+100 | 75 | 38353 | 1890.3 | 100 | 38524 | 1907.4 |
|  |  | 100 | 38376 | 1892.6 | 75 | 38547 | 1909.7 |
| High | 25+100 | 25 | 38433 | 1898.3 | 100 | 38550 | 1910 |
|  |  | 100 | 38500 | 1905.0 | 25 | 38617 | 1916.7 |
|  | 50+100 | 50 | 38406 | 1895.6 | 100 | 38550 | 1910 |
|  |  | 100 | 38451 | 1900.1 | 50 | 38595 | 1914.5 |
|  | 75+100 | 75 | 38379 | 1892.9 | 100 | 38550 | 1910 |
|  |  | 100 | 38401 | 1895.1 | 75 | 38572 | 1912.2 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |

##### 4.3.1.2.8	TDD reference test frequencies for Operating Band 40

Table 4.3.1.2.8-1: Test frequencies for E-UTRA channel bandwidth for operating band 40

| Test Frequency ID | Bandwidth [MHz] | EARFCN | Frequency (UL and DL) [MHz] |
| --- | --- | --- | --- |
| Low Range | 5 | 38675 | 2302.5 |
|  | 10 | 38700 | 2305 |
|  | 15 | 38725 | 2307.5 |
|  | 20 | 38750 | 2310 |
| Mid Range | 5/10/15/20 | 39150 | 2350 |
| High Range | 5 | 39625 | 2397.5 |
|  | 10 | 39600 | 2395 |
|  | 15 | 39575 | 2392.5 |
|  | 20 | 39550 | 2390 |

##### 4.3.1.2.8A	TDD reference test frequencies for CA in operating band 40

Table 4.3.1.2.9A-0: Test frequencies for CA_40A-40A

| Test Frequency ID |  | CC-Combo / NRB_agg [RB] |  | CC1 Note1 |  | Wgap [MHz] |  |  | CC2 Note1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | BW [RB] | NUL/DL | fUL/DL [MHz] |  | BW [RB] |  | NUL/DL | fUL/DL [MHz] |
| Max WGap | 50+50 | 50 | 38700 | 2305 | 80 |  | 50 | 39600 | 2395 |
|  | 50+75 | 50 | 38700 | 2305 | 75 |  | 75 | 39575 | 2392.5 |
|  |  | 75 | 38725 | 2307.5 | 75 |  | 50 | 39600 | 2395 |
|  | 50+100 | 50 | 38700 | 2305 | 70 |  | 100 | 39550 | 2390 |
|  |  | 100 | 38750 | 2310 | 70 |  | 50 | 39600 | 2395 |
|  | 75+75 | 75 | 38725 | 2307.5 | 70 |  | 75 | 39575 | 2392.5 |
|  | 75+100 | 75 | 38725 | 2307.5 | 65 |  | 100 | 39550 | 2390 |
|  |  | 100 | 38750 | 2310 | 65 |  | 75 | 39575 | 2392.5 |
|  | 100+100 | 100 | 38750 | 2310 | 60 |  | 100 | 39550 | 2390 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.8A-1: Test frequencies for CA_40C

| Range | CC-Combo / NRB_agg [RB] |  | CC1 Note1 |  |  | CC2 Note1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL/DL | fUL/DL [MHz] | BW [RB] | NUL/DL | fUL/DL [MHz] |
| Low | 50+100 | 50 | 38705 | 2305.5 | 100 | 38849 | 2319.9 |
|  |  | 100 | 38750 | 2310 | 50 | 38894 | 2324.4 |
|  | 75+75 | 75 | 38725 | 2307.5 | 75 | 38875 | 2322.5 |
|  | 75+100 | 75 | 38728 | 2307.8 | 100 | 38899 | 2324.9 |
|  |  | 100 | 38750 | 2310 | 75 | 38921 | 2327.1 |
|  | 100+100 | 100 | 38750 | 2310 | 100 | 38948 | 2329.8 |
| Mid | 50+100 | 50 | 39056 | 2340.6 | 100 | 39200 | 2355.0 |
|  |  | 100 | 39101 | 2345.1 | 50 | 39245 | 2359.5 |
|  | 75+75 | 75 | 39075 | 2342.5 | 75 | 39225 | 2357.5 |
|  | 75+100 | 75 | 39053 | 2340.3 | 100 | 39224 | 2357.4 |
|  |  | 100 | 39076 | 2342.6 | 75 | 39247 | 2359.7 |
|  | 100+100 | 100 | 39051 | 2340.1 | 100 | 39249 | 2359.9 |
| High | 50+100 | 50 | 39406 | 2375.6 | 100 | 39550 | 2390 |
|  |  | 100 | 39451 | 2380.1 | 50 | 39595 | 2394.5 |
|  | 75+75 | 75 | 39425 | 2377.5 | 75 | 39575 | 2392.5 |
|  | 75+100 | 75 | 39379 | 2372.9 | 100 | 39550 | 2390 |
|  |  | 100 | 39401 | 2375.1 | 75 | 39572 | 2392.2 |
|  | 100+100 | 100 | 39352 | 2370.2 | 100 | 39550 | 2390 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |

Table 4.3.1.2.8A-2: Test frequencies for CA_40D

| Range | CC-Combo / NRB_agg [RB] |  | CC1 Note1 |  |  | CC2 Note1 |  |  | CC3 Note1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL/DL | fUL/DL [MHz] | BW [RB] | NUL/DL | fUL/DL [MHz] | BW [RB] | NUL/DL | fUL/DL [MHz] |
| Low | 50+100+100 | 50 | 38705 | 2305.5 | 100 | 38849 | 2319.9 | 100 | 39047 | 2339.7 |
|  |  | 100 | 38750 | 2310 | 50 | 38894 | 2324.4 | 100 | 39038 | 2338.8 |
|  |  | 100 | 38750 | 2310 | 100 | 38948 | 2329.8 | 50 | 39092 | 2344.2 |
|  | 75+100+100 | 75 | 38728 | 2307.8 | 100 | 38899 | 2324.9 | 100 | 39097 | 2344.7 |
|  |  | 100 | 38750 | 2310 | 75 | 38921 | 2327.1 | 100 | 39092 | 2344.2 |
|  |  | 100 | 38750 | 2310 | 100 | 38948 | 2329.8 | 75 | 39119 | 2346.9 |
|  | 100+100+100 | 100 | 38750 | 2310 | 100 | 38948 | 2329.8 | 100 | 39146 | 2349.6 |
| Mid | 50+100+100 | 50 | 38957 | 2330.7 | 100 | 39101 | 2345.1 | 100 | 39299 | 2364.9 |
|  |  | 100 | 39006 | 2335.6 | 50 | 39150 | 2350 | 100 | 39294 | 2364.4 |
|  |  | 100 | 39002 | 2335.2 | 100 | 39200 | 2355 | 50 | 39344 | 2369.4 |
|  | 75+100+100 | 75 | 38954 | 2330.4 | 100 | 39125 | 2347.5 | 100 | 39323 | 2367.3 |
|  |  | 100 | 38979 | 2332.9 | 75 | 39150 | 2350 | 100 | 39321 | 2367.1 |
|  |  | 100 | 38977 | 2332.7 | 100 | 39175 | 2352.5 | 75 | 39346 | 2369.6 |
|  | 100+100+100 | 100 | 38952 | 2330.2 | 100 | 39150 | 2350 | 100 | 39348 | 2369.8 |
| High | 50+100+100 | 50 | 39208 | 2355.8 | 100 | 39352 | 2370.2 | 100 | 39550 | 2390 |
|  |  | 100 | 39262 | 2361.2 | 50 | 39406 | 2375.6 | 100 | 39550 | 2390 |
|  |  | 100 | 39253 | 2360.3 | 100 | 39451 | 2380.1 | 50 | 39595 | 2394.5 |
|  | 75+100+100 | 75 | 39181 | 2353.1 | 100 | 39352 | 2370.2 | 100 | 39550 | 2390 |
|  |  | 100 | 39208 | 2355.8 | 75 | 39379 | 2372.9 | 100 | 39550 | 2390 |
|  |  | 100 | 39203 | 2355.3 | 100 | 39401 | 2375.1 | 75 | 39572 | 2392.2 |
|  | 100+100+100 | 100 | 39154 | 2350.4 | 100 | 39352 | 2370.2 | 100 | 39550 | 2390 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.8A-3: Test frequencies for CA_40E

| Range | CC-Combo / NRB_agg [RB] |  | CC1 Note1 |  |  | CC2 Note1 |  |  | CC3 Note1 |  |  | CC4 Note1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL/DL | fUL/DL [MHz] | BW [RB] | NUL/DL | fUL/DL [MHz] | BW [RB] | NUL/DL | fUL/DL [MHz] | BW [RB] | NUL/DL | fUL/DL [MHz] |
| Low | 75+75+75+100 | 75 | 38728 | 2307.8 | 75 | 38878 | 2322.8 | 75 | 39028 | 2337.8 | 100 | 39199 | 2354.9 |
|  | 75+75+100+100 | 75 | 38728 | 2307.8 | 75 | 38878 | 2322.8 | 100 | 39049 | 2339.9 | 100 | 39247 | 2359.7 |
|  |  | 75 | 38728 | 2307.8 | 100 | 38899 | 2324.9 | 75 | 39070 | 2342 | 100 | 39241 | 2359.1 |
|  |  | 100 | 38750 | 2310 | 75 | 38921 | 2327.1 | 75 | 39071 | 2342.1 | 100 | 39242 | 2359.2 |
|  | 75+100+100+100 | 75 | 38728 | 2307.8 | 100 | 38899 | 2324.9 | 100 | 39097 | 2344.7 | 100 | 39295 | 2364.5 |
|  |  | 100 | 38750 | 2310 | 75 | 38921 | 2327.1 | 100 | 39092 | 2344.2 | 100 | 39290 | 2364 |
|  |  | 100 | 38750 | 2310 | 100 | 38948 | 2329.8 | 75 | 39119 | 2346.9 | 100 | 39290 | 2364 |
|  | 100+100+100+100 | 100 | 38750 | 2310 | 100 | 38948 | 2329.8 | 100 | 39146 | 2349.6 | 100 | 39344 | 2369.4 |
| Mid | 75+75+75+100 | 75 | 38903 | 2325.3 | 75 | 39053 | 2340.3 | 75 | 39203 | 2355.3 | 100 | 39374 | 2372.4 |
|  | 75+75+100+100 | 75 | 38879 | 2322.9 | 75 | 39029 | 2337.9 | 100 | 39200 | 2355 | 100 | 39398 | 2374.8 |
|  |  | 75 | 38882 | 2323.2 | 100 | 39053 | 2340.3 | 75 | 39224 | 2357.4 | 100 | 39395 | 2374.5 |
|  |  | 100 | 38904 | 2325.4 | 75 | 39075 | 2342.5 | 75 | 39225 | 2357.5 | 100 | 39396 | 2374.6 |
|  | 75+100+100+100 | 75 | 38855 | 2320.5 | 100 | 39026 | 2337.6 | 100 | 39224 | 2357.4 | 100 | 39422 | 2377.2 |
|  |  | 100 | 38880 | 2323 | 75 | 39051 | 2340.1 | 100 | 39222 | 2357.2 | 100 | 39420 | 2377 |
|  |  | 100 | 38880 | 2323 | 100 | 39078 | 2342.8 | 75 | 39249 | 2359.9 | 100 | 39420 | 2377 |
|  | 100+100+100+100 | 100 | 38853 | 2320.3 | 100 | 39051 | 2340.1 | 100 | 39249 | 2359.9 | 100 | 39447 | 2379.7 |
| High | 75+75+75+100 | 75 | 39079 | 2342.9 | 75 | 39229 | 2357.9 | 75 | 39379 | 2372.9 | 100 | 39550 | 2390 |
|  | 75+75+100+100 | 75 | 39031 | 2338.1 | 75 | 39181 | 2353.1 | 100 | 39352 | 2370.2 | 100 | 39550 | 2390 |
|  |  | 75 | 39037 | 2338.7 | 100 | 39208 | 2355.8 | 75 | 39379 | 2372.9 | 100 | 39550 | 2390 |
|  |  | 100 | 39058 | 2340.8 | 75 | 39229 | 2357.9 | 75 | 39379 | 2372.9 | 100 | 39550 | 2390 |
|  | 75+100+100+100 | 75 | 38983 | 2333.3 | 100 | 39154 | 2350.4 | 100 | 39352 | 2370.2 | 100 | 39550 | 2390 |
|  |  | 100 | 39010 | 2336 | 75 | 39181 | 2353.1 | 100 | 39352 | 2370.2 | 100 | 39550 | 2390 |
|  |  | 100 | 39010 | 2336 | 100 | 39208 | 2355.8 | 75 | 39379 | 2372.9 | 100 | 39550 | 2390 |
|  | 100+100+100+100 | 100 | 38956 | 2330.6 | 100 | 39154 | 2350.4 | 100 | 39352 | 2370.2 | 100 | 39550 | 2390 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.8A-4: Test frequencies for CA_40A-40A

| Test Frequency ID | CC-Combo / NRB_agg [RB] |  | CC1 Note1 |  | Wgap [MHz] |  | CC2 Note1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL/DL | fUL/DL [MHz] |  | BW [RB] | NUL/DL | fUL/DL [MHz] |
| Max WGap | 50+50 | 50 | 38700 | 2305 | 80 | 50 | 39600 | 2395 |
|  | 50+75 | 50 | 38700 | 2305 | 75 | 75 | 39575 | 2392.5 |
|  |  | 75 | 38725 | 2307.5 | 75 | 50 | 39600 | 2395 |
|  | 50+100 | 50 | 38700 | 2305 | 70 | 100 | 39550 | 2390 |
|  |  | 100 | 38750 | 2310 | 70 | 50 | 39600 | 2395 |
|  | 75+75 | 75 | 38725 | 2307.5 | 70 | 75 | 39575 | 2392.5 |
|  | 75+100 | 75 | 38725 | 2307.5 | 65 | 100 | 39550 | 2390 |
|  |  | 100 | 38750 | 2310 | 65 | 75 | 39575 | 2392.5 |
|  | 100+100 | 100 | 38750 | 2310 | 60 | 100 | 39550 | 2390 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |

Table 4.3.1.2.8A-5: Test frequencies for CA_40A-40C

| Test Frequency ID | CC-Combo / NRB_agg [RB] |  | CC1 Note1 |  | Wgap [MHz] |  | CC2 Note1 |  |  | CC3 Note1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL/DL | fUL/DL [MHz] |  | BW [RB | NUL/DL | fUL/DL [MHz] | BW [RB] | NUL/DL | fUL/DL [MHz] |
| Max WGap | 100+50+100 | 100 | 38750 | 2310 | 50.6 | 50 | 39406 | 2375.6 | 100 | 39550 | 2390 |
|  |  | 100 | 38750 | 2310 | 50.1 | 100 | 39451 | 2380.1 | 50 | 39595 | 2394.5 |
|  | 100+75+75 | 100 | 38750 | 2310 | 50 | 75 | 39425 | 2377.5 | 75 | 39575 | 2392.5 |
|  | 100+75+100 | 100 | 38750 | 2310 | 45.4 | 75 | 39379 | 2372.9 | 100 | 39550 | 2390 |
|  |  | 100 | 38750 | 2310 | 45.1 | 100 | 39401 | 2375.1 | 75 | 39572 | 2392.2 |
|  | 100+100+100 | 100 | 38750 | 2310 | 40.2 | 100 | 39352 | 2370.2 | 100 | 39550 | 2390 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.8A-6: Test frequencies for CA_40C-40C

| Test frequency ID | CC-Combo / |  | CC1 |  |  | CC2 |  | Wgap |  | CC3 |  |  | CC4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB_agg |  | Note1 |  |  | Note1 |  | [MHz] |  | Note1 |  |  | Note1 |  |
|  | [RB] | BW | NUL/DL | fUL/DL | BW | NUL/DL | fUL/DL |  | BW | NUL/DL | fUL/DL | BW | NUL/DL | fUL/DL |
|  |  | [RB] |  | [MHz] | [RB] |  | [MHz] |  | [RB |  | [MHz] | [RB] |  | [MHz] |
| Max Wgap | 50+50+ 100+100 | 50 | 38705 | 2305.5 | 100 | 38849 | 2319.9 | 40.7 | 50 | 39406 | 2375.6 | 100 | 39550 | 2390 |
|  |  | 100 | 38750 | 2310 | 50 | 38894 | 2324.4 | 40.7 | 100 | 39451 | 2380.1 | 50 | 39595 | 2394.5 |
|  |  | 50 | 38705 | 2305.5 | 100 | 38849 | 2319.9 | 40.2 | 100 | 39451 | 2380.1 | 50 | 39595 | 2394.5 |
|  |  | 100 | 38750 | 2310 | 50 | 38894 | 2324.4 | 41.2 | 50 | 39406 | 2375.6 | 100 | 39550 | 2390 |
|  | 50+75+75 100 | 50 | 38705 | 2305.5 | 100 | 38849 | 2319.9 | 40.1 | 75 | 39425 | 2377.5 | 75 | 39575 | 2392.5 |
|  |  | 100 | 38750 | 2310 | 50 | 38894 | 2324.4 | 40.6 | 75 | 39425 | 2377.5 | 75 | 39575 | 2392.5 |
|  |  | 75 | 38725 | 2307.5 | 75 | 38875 | 2322.5 | 40.6 | 50 | 39406 | 2375.6 | 100 | 39550 | 2390 |
|  |  | 75 | 38725 | 2307.5 | 75 | 38875 | 2322.5 | 40.1 | 100 | 39451 | 2380.1 | 50 | 39595 | 2394.5 |
|  | 75+75+ 75+75 | 75 | 38725 | 2307.5 | 75 | 38875 | 2322.5 | 40 | 75 | 39425 | 2377.5 | 75 | 39575 | 2392.5 |
|  | 75+75+ 75+100 | 75 | 38725 | 2307.5 | 75 | 38875 | 2322.5 | 35.4 | 75 | 39379 | 2372.9 | 100 | 39550 | 2390 |
|  |  | 75 | 38725 | 2307.5 | 75 | 38875 | 2322.5 | 35.1 | 100 | 39401 | 2375.1 | 75 | 39572 | 2392.2 |
|  |  | 75 | 38728 | 2307.8 | 100 | 38899 | 2324.9 | 35.1 | 75 | 39425 | 2377.5 | 75 | 39575 | 2392.5 |
|  |  | 100 | 38750 | 2310 | 75 | 38921 | 2327.1 | 35.4 | 75 | 39425 | 2377.5 | 75 | 39575 | 2392.5 |
|  | 50+75+ 100+100 | 50 | 38705 | 2305.5 | 100 | 38849 | 2319.9 | 35.5 | 75 | 39379 | 2372.9 | 100 | 39550 | 2390 |
|  |  | 50 | 38705 | 2305.5 | 100 | 38849 | 2319.9 | 35.2 | 100 | 39401 | 2375.1 | 75 | 39572 | 2392.2 |
|  |  | 75 | 38728 | 2307.8 | 100 | 38899 | 2324.9 | 35.7 | 50 | 39406 | 2375.6 | 100 | 39550 | 2390 |
|  |  | 75 | 38728 | 2307.8 | 100 | 38899 | 2324.9 | 35.2 | 100 | 39451 | 2380.1 | 50 | 39595 | 2394.5 |
|  |  | 100 | 38750 | 2310 | 50 | 38894 | 2324.4 | 36 | 75 | 39379 | 2372.9 | 100 | 39550 | 2390 |
|  |  | 100 | 38750 | 2310 | 50 | 38894 | 2324.4 | 35.7 | 100 | 39401 | 2375.1 | 75 | 39572 | 2392.2 |
|  |  | 100 | 38750 | 2310 | 75 | 38921 | 2327.1 | 36 | 50 | 39406 | 2375.6 | 100 | 39550 | 2390 |
|  |  | 100 | 38750 | 2310 | 75 | 38921 | 2327.1 | 35.5 | 100 | 39451 | 2380.1 | 50 | 39595 | 2394.5 |
|  | 50+100+ 100+100 | 50 | 38705 | 2305.5 | 100 | 38849 | 2319.9 | 30.3 | 100 | 39352 | 2370.2 | 100 | 39550 | 2390 |
|  |  | 100 | 38750 | 2310 | 50 | 38894 | 2324.4 | 30.8 | 100 | 39352 | 2370.2 | 100 | 39550 | 2390 |
|  |  | 100 | 38750 | 2310 | 100 | 38948 | 2329.8 | 30.8 | 50 | 39406 | 2375.6 | 100 | 39550 | 2390 |
|  |  | 100 | 38750 | 2310 | 100 | 38948 | 2329.8 | 30.3 | 100 | 39451 | 2380.1 | 50 | 39595 | 2394.5 |
|  | 75+75+ 100+100 | 75 | 38725 | 2307.5 | 75 | 38875 | 2322.5 | 30.2 | 100 | 39352 | 2370.2 | 100 | 39550 | 2390 |
|  |  | 100 | 38750 | 2310 | 100 | 38948 | 2329.8 | 30.2 | 75 | 39425 | 2377.5 | 75 | 39575 | 2392.5 |
|  |  | 75 | 38728 | 2307.8 | 100 | 38899 | 2324.9 | 30.5 | 75 | 39379 | 2372.9 | 100 | 39550 | 2390 |
|  |  | 75 | 38728 | 2307.8 | 100 | 38899 | 2324.9 | 30.2 | 100 | 39401 | 2375.1 | 75 | 39572 | 2392.2 |
|  |  | 100 | 38750 | 2310 | 75 | 38921 | 2327.1 | 30.8 | 75 | 39379 | 2372.9 | 100 | 39550 | 2390 |
|  |  | 100 | 38750 | 2310 | 75 | 38921 | 2327.1 | 30.5 | 100 | 39401 | 2375.1 | 75 | 39572 | 2392.2 |
|  | 75+100+ 100+100 | 75 | 38728 | 2307.8 | 100 | 38899 | 2324.9 | 25.3 | 100 | 39352 | 2370.2 | 100 | 39550 | 2390 |
|  |  | 100 | 38750 | 2310 | 75 | 38921 | 2327.1 | 25.6 | 100 | 39352 | 2370.2 | 100 | 39550 | 2390 |
|  |  | 100 | 38750 | 2310 | 100 | 38948 | 2329.8 | 25.6 | 75 | 39379 | 2372.9 | 100 | 39550 | 2390 |
|  |  | 100 | 38750 | 2310 | 100 | 38948 | 2329.8 | 25.3 | 100 | 39401 | 2375.1 | 75 | 39572 | 2392.2 |
|  | 100+100+ 100+100 | 100 | 38750 | 2310 | 100 | 38948 | 2329.8 | 20.4 | 100 | 39352 | 2370.2 | 100 | 39550 | 2390 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### 4.3.1.2.9	TDD reference test frequencies for Operating Band 41

Table 4.3.1.2.9-1: Test frequencies for E-UTRA channel bandwidth for operating band 41

| Test Frequency ID | Bandwidth [MHz] | EARFCN | Frequency (UL and DL) [MHz] |
| --- | --- | --- | --- |
| Low Range | 5 | 39675 | 2498.5 |
|  | 10 | 39700 | 2501 |
|  | 15 | 39725 | 2503.5 |
|  | 20 | 39750 | 2506 |
| Mid Range | 5/10/15/20 | 40620 | 2593 |
| High Range | 5 | 41565 | 2687.5 |
|  | 10 | 41540 | 2685 |
|  | 15 | 41515 | 2682.5 |
|  | 20 | 41490 | 2680 |

##### 4.3.1.2.9A	TDD reference test frequencies for CA in operating band 41

Table 4.3.1.2.9A-1: Test frequencies for CA_41C

| Range | CC-Combo / NRB_agg [RB] |  | CC1 Note1 |  |  | CC2 Note1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL/DL | fUL/DL [MHz] | BW [RB] | NUL/DL | fUL/DL [MHz] |
| Low | 25+100 | 25 | 39683 | 2499.3 | 100 | 39800 | 2511 |
|  |  | 100 | 39750 | 2506 | 25 | 39867 | 2517.7 |
|  | 50+75 | 50 | 39703 | 2501.3 | 75 | 39823 | 2513.3 |
|  |  | 75 | 39725 | 2503.5 | 50 | 39845 | 2515.5 |
|  | 50+100 | 50 | 39705 | 2501.5 | 100 | 39849 | 2515.9 |
|  |  | 100 | 39750 | 2506 | 50 | 39894 | 2520.4 |
|  | 75+75 | 75 | 39725 | 2503.5 | 75 | 39875 | 2518.5 |
|  | 75+100 | 75 | 39728 | 2503.8 | 100 | 39899 | 2520.9 |
|  |  | 100 | 39750 | 2506 | 75 | 39921 | 2523.1 |
|  | 100+100 | 100 | 39750 | 2506 | 100 | 39948 | 2525.8 |
| Mid | 25+100 | 25 | 40528 | 2583.8 | 100 | 40645 | 2595.5 |
|  |  | 100 | 40595 | 2590.5 | 25 | 40712 | 2602.2 |
|  | 50+75 | 50 | 40549 | 2585.9 | 75 | 40669 | 2597.9 |
|  |  | 75 | 40571 | 2588.1 | 50 | 40691 | 2600.1 |
|  | 50+100 | 50 | 40526 | 2583.6 | 100 | 40670 | 2598.0 |
|  |  | 100 | 40571 | 2588.1 | 50 | 40715 | 2602.5 |
|  | 75+75 | 75 | 40545 | 2585.5 | 75 | 40695 | 2600.5 |
|  | 75+100 | 75 | 40523 | 2583.3 | 100 | 40694 | 2600.4 |
|  |  | 100 | 40546 | 2585.6 | 75 | 40717 | 2602.7 |
|  | 100+100 | 100 | 40521 | 2583.1 | 100 | 40719 | 2602.9 |
|  | 100+1002 | 100 | 40529 | 2583.9 | 100 | 40712 | 2602.2 |
| High | 25+100 | 25 | 41373 | 2668.3 | 100 | 41490 | 2680 |
|  |  | 100 | 41440 | 2675 | 25 | 41557 | 2686.7 |
|  | 50+75 | 50 | 41395 | 2670.5 | 75 | 41515 | 2682.5 |
|  |  | 75 | 41417 | 2672.7 | 50 | 41537 | 2684.7 |
|  | 50+100 | 50 | 41346 | 2665.6 | 100 | 41490 | 2680 |
|  |  | 100 | 41391 | 2670.1 | 50 | 41535 | 2684.5 |
|  | 75+75 | 75 | 41365 | 2667.5 | 75 | 41515 | 2682.5 |
|  | 75+100 | 75 | 41319 | 2662.9 | 100 | 41490 | 2680 |
|  |  | 100 | 41341 | 2665.1 | 75 | 41512 | 2682.2 |
|  | 100+100 | 100 | 41292 | 2660.2 | 100 | 41490 | 2680 |
| Note 1: Carriers in increasing frequency order. Note 2: This test frequency is applicable only for intra-band contiguous CA which requires channel spacing to be less than norminal channel spacing. |  |  |  |  |  |  |  |

Table 4.3.1.2.9A-2: Test frequencies for CA_41A-41A

| Test Frequency ID | CC-Combo / NRB_agg [RB] |  | CC1 Note1 |  | Wgap [MHz] |  | CC2 Note1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL/DL | fUL/DL [MHz] |  | BW [RB] | NUL/DL | fUL/DL [MHz] |
| Max WGap | 25+25 | 25 | 39675 | 2498.5 | 184 | 25 | 41565 | 2687.5 |
|  | 25+50 | 25 | 39675 | 2498.5 | 179 | 50 | 41540 | 2685 |
|  |  | 50 | 39700 | 2501 | 179 | 25 | 41565 | 2687.5 |
|  | 25+75 | 25 | 39675 | 2498.5 | 174 | 75 | 41515 | 2682.5 |
|  |  | 75 | 39725 | 2503.5 | 174 | 25 | 41565 | 2687.5 |
|  | 50+50 | 50 | 39700 | 2501 | 174 | 50 | 41540 | 2685 |
|  | 25+100 | 25 | 39675 | 2498.5 | 169 | 100 | 41490 | 2680 |
|  |  | 100 | 39750 | 2506 | 169 | 25 | 41565 | 2687.5 |
|  | 50+75 | 50 | 39700 | 2501 | 169 | 75 | 41515 | 2682.5 |
|  |  | 75 | 39725 | 2503.5 | 169 | 50 | 41540 | 2685 |
|  | 50+100 | 50 | 39700 | 2501 | 164 | 100 | 41490 | 2680 |
|  |  | 100 | 39750 | 2506 | 164 | 50 | 41540 | 2685 |
|  | 75+75 | 75 | 39725 | 2503.5 | 164 | 75 | 41515 | 2682.5 |
|  | 75+100 | 75 | 39725 | 2503.5 | 159 | 100 | 41490 | 2680 |
|  |  | 100 | 39750 | 2506 | 159 | 75 | 41515 | 2682.5 |
|  | 100+100 | 100 | 39750 | 2506 | 154 | 100 | 41490 | 2680 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |

Table 4.3.1.2.9A-3: Test frequencies for CA_41D

| Range | CC-Combo / NRB_agg [RB] |  | CC1 Note1 |  |  | CC2 Note1 |  |  | CC3 Note1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL/DL | fUL/DL [MHz] | BW [RB] | NUL/DL | fUL/DL [MHz] | BW [RB] | NUL/DL | fUL/DL [MHz] |
| Low | 50+75+100 | 50 | 39705 | 2501.5 | 75 | 39825 | 2513.5 | 100 | 39996 | 2530.6 |
|  |  | 50 | 39705 | 2501.5 | 100 | 39849 | 2515.9 | 75 | 40020 | 2533 |
|  |  | 75 | 39728 | 2503.8 | 50 | 39848 | 2515.8 | 100 | 39992 | 2530.2 |
|  |  | 75 | 39728 | 2503.8 | 100 | 39899 | 2520.9 | 50 | 40043 | 2535.3 |
|  |  | 100 | 39750 | 2506 | 50 | 39894 | 2520.4 | 75 | 40014 | 2532.4 |
|  |  | 100 | 39750 | 2506 | 75 | 39921 | 2523.1 | 50 | 40041 | 2535.1 |
|  | 50+100+100 | 50 | 39705 | 2501.5 | 100 | 39849 | 2515.9 | 100 | 40047 | 2535.7 |
|  |  | 100 | 39750 | 2506 | 50 | 39894 | 2520.4 | 100 | 40038 | 2534.8 |
|  |  | 100 | 39750 | 2506 | 100 | 39948 | 2525.8 | 50 | 40092 | 2540.2 |
|  | 75+75+100 | 75 | 39728 | 2503.8 | 75 | 39878 | 2518.8 | 100 | 40049 | 2535.9 |
|  |  | 75 | 39728 | 2503.8 | 100 | 39899 | 2520.9 | 75 | 40070 | 2538 |
|  |  | 100 | 39750 | 2506 | 75 | 39921 | 2523.1 | 75 | 40071 | 2538.1 |
|  | 75+100+100 | 75 | 39728 | 2503.8 | 100 | 39899 | 2520.9 | 100 | 40097 | 2540.7 |
|  |  | 100 | 39750 | 2506 | 75 | 39921 | 2523.1 | 100 | 40092 | 2540.2 |
|  |  | 100 | 39750 | 2506 | 100 | 39948 | 2525.8 | 75 | 40119 | 2542.9 |
|  | 100+100+100 | 100 | 39750 | 2506 | 100 | 39948 | 2525.8 | 100 | 40146 | 2545.6 |
| Mid | 50+75+100 | 50 | 40452 | 2576.2 | 75 | 40572 | 2588.2 | 100 | 40743 | 2605.3 |
|  |  | 50 | 40451 | 2576.1 | 100 | 40595 | 2590.5 | 75 | 40766 | 2607.6 |
|  |  | 75 | 40477 | 2578.7 | 50 | 40597 | 2590.7 | 100 | 40741 | 2605.1 |
|  |  | 75 | 40474 | 2578.4 | 100 | 40645 | 2595.5 | 50 | 40789 | 2609.9 |
|  |  | 100 | 40499 | 2580.9 | 50 | 40643 | 2595.3 | 75 | 40763 | 2607.3 |
|  |  | 100 | 40497 | 2580.7 | 75 | 40668 | 2597.8 | 50 | 40788 | 2609.8 |
|  | 50+100+100 | 50 | 40427 | 2573.7 | 100 | 40571 | 2588.1 | 100 | 40769 | 2607.9 |
|  |  | 100 | 40476 | 2578.6 | 50 | 40620 | 2593 | 100 | 40764 | 2607.4 |
|  |  | 100 | 40472 | 2578.2 | 100 | 40670 | 2598 | 50 | 40814 | 2612.4 |
|  | 75+75+100 | 75 | 40448 | 2575.8 | 75 | 40598 | 2590.8 | 100 | 40769 | 2607.9 |
|  |  | 75 | 40449 | 2575.9 | 100 | 40620 | 2593 | 75 | 40791 | 2610.1 |
|  |  | 100 | 40471 | 2578.1 | 75 | 40642 | 2595.2 | 75 | 40792 | 2610.2 |
|  | 75+100+100 | 75 | 40424 | 2573.4 | 100 | 40595 | 2590.5 | 100 | 40793 | 2610.3 |
|  |  | 100 | 40449 | 2575.9 | 75 | 40620 | 2593 | 100 | 40791 | 2610.1 |
|  |  | 100 | 40447 | 2575.7 | 100 | 40645 | 2595.5 | 75 | 40816 | 2612.6 |
|  | 100+100+100 | 100 | 40422 | 2573.2 | 100 | 40620 | 2593 | 100 | 40818 | 2612.8 |
|  | 100+100+1002 | 100 | 40437 | 2574.7 | 100 | 40620 | 2593 | 100 | 40803 | 2611.3 |
| High | 50+75+100 | 50 | 41199 | 2650.9 | 75 | 41319 | 2662.9 | 100 | 41490 | 2680 |
|  |  | 50 | 41197 | 2650.7 | 100 | 41341 | 2665.1 | 75 | 41512 | 2682.2 |
|  |  | 75 | 41226 | 2653.6 | 50 | 41346 | 2665.6 | 100 | 41490 | 2680 |
|  |  | 75 | 41220 | 2653 | 100 | 41391 | 2670.1 | 50 | 41535 | 2684.5 |
|  |  | 100 | 41248 | 2655.8 | 50 | 41392 | 2670.2 | 75 | 41512 | 2682.2 |
|  |  | 100 | 41244 | 2655.4 | 75 | 41415 | 2672.5 | 50 | 41535 | 2684.5 |
|  | 50+100+100 | 50 | 41148 | 2645.8 | 100 | 41292 | 2660.2 | 100 | 41490 | 2680 |
|  |  | 100 | 41202 | 2651.2 | 50 | 41346 | 2665.6 | 100 | 41490 | 2680 |
|  |  | 100 | 41193 | 2650.3 | 100 | 41391 | 2670.1 | 50 | 41535 | 2684.5 |
|  | 75+75+100 | 75 | 41169 | 2647.9 | 75 | 41319 | 2662.9 | 100 | 41490 | 2680 |
|  |  | 75 | 41170 | 2648 | 100 | 41341 | 2665.1 | 75 | 41512 | 2682.2 |
|  |  | 100 | 41191 | 2650.1 | 75 | 41362 | 2667.2 | 75 | 41512 | 2682.2 |
|  | 75+100+100 | 75 | 41121 | 2643.1 | 100 | 41292 | 2660.2 | 100 | 41490 | 2680 |
|  |  | 100 | 41148 | 2645.8 | 75 | 41319 | 2662.9 | 100 | 41490 | 2680 |
|  |  | 100 | 41143 | 2645.3 | 100 | 41341 | 2665.1 | 75 | 41512 | 2682.2 |
|  | 100+100+100 | 100 | 41094 | 2640.4 | 100 | 41292 | 2660.2 | 100 | 41490 | 2680 |
| Note 1: Carriers in increasing frequency order. Note 2: This test frequency is applicable only for intra-band contiguous CA which requires channel spacing to be less than norminal channel spacing. |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.9A-4: Test frequencies for CA_41C-41A

| Test Frequency ID | CC-Combo / NRB_agg [RB] |  | CC1 Note1 |  |  | CC2 Note1 |  | Wgap [MHz] |  | CC3 Note1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL/DL | fUL/DL [MHz] | BW [RB | NUL/DL | fUL/DL [MHz] |  | BW [RB] | NUL/DL | fUL/DL [MHz] |
| Max WGap | 25+25+100 | 25 | 39683 | 2499.3 | 100 | 39800 | 2511 | 164 | 25 | 41565 | 2687.5 |
|  |  | 100 | 39750 | 2506 | 25 | 39867 | 2517.7 | 164.05 | 25 | 41565 | 2687.5 |
|  | 25+50+100 | 25 | 39683 | 2499.3 | 100 | 39800 | 2511 | 159 | 50 | 41540 | 2685 |
|  |  | 50 | 39705 | 2501.5 | 100 | 39849 | 2515.9 | 159.1 | 25 | 41565 | 2687.5 |
|  |  | 100 | 39750 | 2506 | 25 | 39867 | 2517.7 | 159.05 | 50 | 41540 | 2685 |
|  |  | 100 | 39750 | 2506 | 50 | 39894 | 2520.4 | 159.1 | 25 | 41565 | 2687.5 |
|  | 25+75+75 | 75 | 39725 | 2503.5 | 75 | 39875 | 2518.5 | 159 | 25 | 41565 | 2687.5 |
|  | 25+75+100 | 25 | 39683 | 2499.3 | 100 | 39800 | 2511 | 154 | 75 | 41515 | 2682.5 |
|  |  | 75 | 39728 | 2503.8 | 100 | 39899 | 2520.9 | 154.1 | 25 | 41565 | 2687.5 |
|  |  | 100 | 39750 | 2506 | 25 | 39867 | 2517.7 | 154.05 | 75 | 41515 | 2682.5 |
|  |  | 100 | 39750 | 2506 | 75 | 39921 | 2523.1 | 154.15 | 25 | 41565 | 2687.5 |
|  | 50+50+100 | 50 | 39705 | 2501.5 | 100 | 39849 | 2515.9 | 154.1 | 50 | 41540 | 2685 |
|  |  | 100 | 39750 | 2506 | 50 | 39894 | 2520.4 | 154.1 | 50 | 41540 | 2685 |
|  | 50+75+75 | 75 | 39725 | 2503.5 | 75 | 39875 | 2518.5 | 154 | 50 | 41540 | 2685 |
|  | 25+100+100 | 25 | 39683 | 2499.3 | 100 | 39800 | 2511 | 149 | 100 | 41490 | 2680 |
|  |  | 100 | 39750 | 2506 | 25 | 39867 | 2517.7 | 149.05 | 100 | 41490 | 2680 |
|  |  | 100 | 39750 | 2506 | 100 | 39948 | 2525.8 | 149.2 | 25 | 41565 | 2687.5 |
|  | 50+75+100 | 50 | 39705 | 2501.5 | 100 | 39849 | 2515.9 | 149.1 | 75 | 41515 | 2682.5 |
|  |  | 75 | 39728 | 2503.8 | 100 | 39899 | 2520.9 | 149.1 | 50 | 41540 | 2685 |
|  |  | 100 | 39750 | 2506 | 50 | 39894 | 2520.4 | 149.1 | 75 | 41515 | 2682.5 |
|  |  | 100 | 39750 | 2506 | 75 | 39921 | 2523.1 | 149.15 | 50 | 41540 | 2685 |
|  | 75+75+75 | 75 | 39725 | 2503.5 | 75 | 39875 | 2518.5 | 149 | 75 | 41515 | 2682.5 |
|  | 50+100+100 | 50 | 39705 | 2501.5 | 100 | 39849 | 2515.9 | 144.1 | 100 | 41490 | 2680 |
|  |  | 100 | 39750 | 2506 | 50 | 39894 | 2520.4 | 144.1 | 100 | 41490 | 2680 |
|  |  | 100 | 39750 | 2506 | 100 | 39948 | 2525.8 | 144.2 | 50 | 41540 | 2685 |
|  | 75+75+100 | 75 | 39725 | 2503.5 | 75 | 39875 | 2518.5 | 144 | 100 | 41490 | 2680 |
|  |  | 75 | 39728 | 2503.8 | 100 | 39899 | 2520.9 | 144.1 | 75 | 41515 | 2682.5 |
|  |  | 100 | 39750 | 2506 | 75 | 39921 | 2523.1 | 144.15 | 75 | 41515 | 2682.5 |
|  | 75+100+100 | 75 | 39728 | 2503.8 | 100 | 39899 | 2520.9 | 139.1 | 100 | 41490 | 2680 |
|  |  | 100 | 39750 | 2506 | 75 | 39921 | 2523.1 | 139.15 | 100 | 41490 | 2680 |
|  |  | 100 | 39750 | 2506 | 100 | 39948 | 2525.8 | 139.2 | 75 | 41515 | 2682.5 |
|  | 100+100+100 | 100 | 39750 | 2506 | 100 | 39948 | 2525.8 | 134.2 | 100 | 41490 | 2680 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.9A-5: Test frequencies for CA_41A-41C

| Test Frequency ID | CC-Combo / NRB_agg [RB] |  | CC1 Note1 |  | Wgap [MHz] |  | CC2 Note1 |  |  | CC3 Note1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL/DL | fUL/DL [MHz] |  | BW [RB | NUL/DL | fUL/DL [MHz] | BW [RB] | NUL/DL | fUL/DL [MHz] |
| Max WGap | 25+25+100 | 25 | 39675 | 2498.5 | 164.05 | 25 | 41373 | 2668.3 | 100 | 41490 | 2680 |
|  |  | 25 | 39675 | 2498.5 | 164 | 100 | 41440 | 2675 | 25 | 41557 | 2686.7 |
|  | 25+50+100 | 25 | 39675 | 2498.5 | 159.1 | 50 | 41346 | 2665.6 | 100 | 41490 | 2680 |
|  |  | 25 | 39675 | 2498.5 | 159.1 | 100 | 41391 | 2670.1 | 50 | 41535 | 2684.5 |
|  |  | 50 | 39700 | 2501 | 159.05 | 25 | 41373 | 2668.3 | 100 | 41490 | 2680 |
|  |  | 50 | 39700 | 2501 | 159 | 100 | 41440 | 2675 | 25 | 41557 | 2686.7 |
|  | 25+75+75 | 25 | 39675 | 2498.5 | 159 | 75 | 41365 | 2667.5 | 75 | 41515 | 2682.5 |
|  | 25+75+100 | 25 | 39675 | 2498.5 | 154.15 | 75 | 41319 | 2662.9 | 100 | 41490 | 2680 |
|  |  | 25 | 39675 | 2498.5 | 154.1 | 100 | 41341 | 2665.1 | 75 | 41512 | 2682.2 |
|  |  | 75 | 39725 | 2503.5 | 154.05 | 25 | 41373 | 2668.3 | 100 | 41490 | 2680 |
|  |  | 75 | 39725 | 2503.5 | 154 | 100 | 41440 | 2675 | 25 | 41557 | 2686.7 |
|  | 50+50+100 | 50 | 39700 | 2501 | 154.1 | 50 | 41346 | 2665.6 | 100 | 41490 | 2680 |
|  |  | 50 | 39700 | 2501 | 154.1 | 100 | 41391 | 2670.1 | 50 | 41535 | 2684.5 |
|  | 25+100+100 | 25 | 39675 | 2498.5 | 149.2 | 100 | 41292 | 2660.2 | 100 | 41490 | 2680 |
|  |  | 100 | 39750 | 2506 | 149.05 | 25 | 41373 | 2668.3 | 100 | 41490 | 2680 |
|  |  | 100 | 39750 | 2506 | 149 | 100 | 41440 | 2675 | 25 | 41557 | 2686.7 |
|  | 50+75+75 | 50 | 39700 | 2501 | 154 | 75 | 41365 | 2667.5 | 75 | 41515 | 2682.5 |
|  | 50+75+100 | 50 | 39700 | 2501 | 149.15 | 75 | 41319 | 2662.9 | 100 | 41490 | 2680 |
|  |  | 50 | 39700 | 2501 | 149.1 | 100 | 41341 | 2665.1 | 75 | 41512 | 2682.2 |
|  |  | 75 | 39725 | 2503.5 | 149.1 | 50 | 41346 | 2665.6 | 100 | 41490 | 2680 |
|  |  | 75 | 39725 | 2503.5 | 149.1 | 100 | 41391 | 2670.1 | 50 | 41535 | 2684.5 |
|  | 75+75+75 | 75 | 39725 | 2503.5 | 149 | 75 | 41365 | 2667.5 | 75 | 41515 | 2682.5 |
|  | 50+100+100 | 50 | 39700 | 2501 | 144.2 | 100 | 41292 | 2660.2 | 100 | 41490 | 2680 |
|  |  | 100 | 39750 | 2506 | 144.1 | 50 | 41346 | 2665.6 | 100 | 41490 | 2680 |
|  |  | 100 | 39750 | 2506 | 144.1 | 100 | 41391 | 2670.1 | 50 | 41535 | 2684.5 |
|  | 75+75+100 | 75 | 39725 | 2503.5 | 144.15 | 75 | 41319 | 2662.9 | 100 | 41490 | 2680 |
|  |  | 75 | 39725 | 2503.5 | 144.1 | 100 | 41341 | 2665.1 | 75 | 41512 | 2682.2 |
|  |  | 100 | 39750 | 2506 | 144 | 75 | 41365 | 2667.5 | 75 | 41515 | 2682.5 |
|  | 75+100+100 | 75 | 39725 | 2503.5 | 139.2 | 100 | 41292 | 2660.2 | 100 | 41490 | 2680 |
|  |  | 100 | 39750 | 2506 | 139.15 | 75 | 41319 | 2662.9 | 100 | 41490 | 2680 |
|  |  | 100 | 39750 | 2506 | 139.1 | 100 | 41341 | 2665.1 | 75 | 41512 | 2682.2 |
|  | 100+100+100 | 100 | 39750 | 2506 | 134.2 | 100 | 41292 | 2660.2 | 100 | 41490 | 2680 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.9A-6: Test frequencies for CA_41C-41C

| Test frequency ID | CC-Combo / |  | CC1 |  |  | CC2 |  | Wgap |  | CC3 |  |  | CC4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB_agg |  | Note1 |  |  | Note1 |  | [MHz] |  | Note1 |  |  | Note1 |  |
|  | [RB] | BW | NUL/DL | fUL/DL | BW | NUL/DL | fUL/DL |  | BW | NUL/DL | fUL/DL | BW | NUL/DL | fUL/DL |
|  |  | [RB] |  | [MHz] | [RB] |  | [MHz] |  | [RB |  | [MHz] | [RB] |  | [MHz] |
| Max Wgap | 50+50+ 100+100 | 50 | 39705 | 2501.5 | 100 | 39849 | 2515.9 | 134.7 | 50 | 41346 | 2665.6 | 100 | 41490 | 2680 |
|  |  | 100 | 39750 | 2506 | 50 | 39894 | 2520.4 | 134.7 | 100 | 41391 | 2670.1 | 50 | 41535 | 2684.5 |
|  |  | 50 | 39705 | 2501.5 | 100 | 39849 | 2515.9 | 134.2 | 100 | 41391 | 2670.1 | 50 | 41535 | 2684.5 |
|  |  | 100 | 39750 | 2506 | 50 | 39894 | 2520.4 | 135.2 | 50 | 41346 | 2665.6 | 100 | 41490 | 2680 |
|  | 50+75+75 100 | 50 | 39705 | 2501.5 | 100 | 39849 | 2515.9 | 134.1 | 75 | 41365 | 2667.5 | 75 | 41515 | 2682.5 |
|  |  | 100 | 39750 | 2506 | 50 | 39894 | 2520.4 | 134.6 | 75 | 41365 | 2667.5 | 75 | 41515 | 2682.5 |
|  |  | 75 | 39725 | 2503.5 | 75 | 39875 | 2518.5 | 134.6 | 50 | 41346 | 2665.6 | 100 | 41490 | 2680 |
|  |  | 75 | 39725 | 2503.5 | 75 | 39875 | 2518.5 | 134.1 | 100 | 41391 | 2670.1 | 50 | 41535 | 2684.5 |
|  | 50+75+ 100+100 | 50 | 39705 | 2501.5 | 100 | 39849 | 2515.9 | 129.5 | 75 | 41319 | 2662.9 | 100 | 41490 | 2680 |
|  |  | 100 | 39750 | 2506 | 50 | 39894 | 2520.4 | 129.7 | 100 | 41341 | 2665.1 | 75 | 41512 | 2682.2 |
|  |  | 50 | 39705 | 2501.5 | 100 | 39849 | 2515.9 | 129.2 | 100 | 41341 | 2665.1 | 75 | 41512 | 2682.2 |
|  |  | 100 | 39750 | 2506 | 50 | 39894 | 2520.4 | 130 | 75 | 41319 | 2662.9 | 100 | 41490 | 2680 |
|  |  | 75 | 40523 | 2583.3 | 100 | 40694 | 2600.4 | 50.2 | 50 | 41346 | 2665.6 | 100 | 41490 | 2680 |
|  |  | 100 | 39750 | 2506 | 75 | 39921 | 2523.1 | 129.5 | 100 | 41391 | 2670.1 | 50 | 41535 | 2684.5 |
|  |  | 100 | 39750 | 2506 | 75 | 39921 | 2523.1 | 130 | 50 | 41346 | 2665.6 | 100 | 41490 | 2680 |
|  |  | 75 | 40523 | 2583.3 | 100 | 40694 | 2600.4 | 49.7 | 100 | 41391 | 2670.1 | 50 | 41535 | 2684.5 |
|  | 50+100+ 100+100 | 50 | 39705 | 2501.5 | 100 | 39849 | 2515.9 | 124.3 | 100 | 41292 | 2660.2 | 100 | 41490 | 2680 |
|  |  | 100 | 39750 | 2506 | 50 | 39894 | 2520.4 | 124.8 | 100 | 41292 | 2660.2 | 100 | 41490 | 2680 |
|  |  | 100 | 39750 | 2506 | 100 | 39948 | 2525.8 | 124.8 | 50 | 41346 | 2665.6 | 100 | 41490 | 2680 |
|  |  | 100 | 39750 | 2506 | 100 | 39948 | 2525.8 | 124.3 | 100 | 41391 | 2670.1 | 50 | 41535 | 2684.5 |
|  | 75+75+ 75+100 | 75 | 40523 | 2583.3 | 100 | 40694 | 2600.4 | 49.6 | 75 | 41365 | 2667.5 | 75 | 41515 | 2682.5 |
|  |  | 100 | 39750 | 2506 | 75 | 39921 | 2523.1 | 129.4 | 75 | 41365 | 2667.5 | 75 | 41515 | 2682.5 |
|  |  | 75 | 39725 | 2503.5 | 75 | 39875 | 2518.5 | 129.4 | 75 | 41319 | 2662.9 | 100 | 41490 | 2680 |
|  |  | 75 | 39725 | 2503.5 | 75 | 39875 | 2518.5 | 129.1 | 100 | 41341 | 2665.1 | 75 | 41512 | 2682.2 |
|  | 75+75+ 100+100 | 75 | 39725 | 2503.5 | 75 | 39875 | 2518.5 | 124.2 | 100 | 41292 | 2660.2 | 100 | 41490 | 2680 |
|  |  | 100 | 39750 | 2506 | 100 | 39948 | 2525.8 | 124.2 | 75 | 41365 | 2667.5 | 75 | 41515 | 2682.5 |
|  | 75+100+ 100+100 | 75 | 40523 | 2583.3 | 100 | 40694 | 2600.4 | 39.8 | 100 | 41292 | 2660.2 | 100 | 41490 | 2680 |
|  |  | 100 | 39750 | 2506 | 75 | 39921 | 2523.1 | 119.6 | 100 | 41292 | 2660.2 | 100 | 41490 | 2680 |
|  |  | 100 | 39750 | 2506 | 100 | 39948 | 2525.8 | 119.6 | 75 | 41319 | 2662.9 | 100 | 41490 | 2680 |
|  |  | 100 | 39750 | 2506 | 100 | 39948 | 2525.8 | 119.3 | 100 | 41341 | 2665.1 | 75 | 41512 | 2682.2 |
|  | 100+100+ 100+100 | 100 | 39750 | 2506 | 100 | 39948 | 2525.8 | 114.4 | 100 | 41292 | 2660.2 | 100 | 41490 | 2680 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.9A-7: Test frequencies for CA_41A-41D

| Test frequency ID | CC-Combo / |  | CC1 |  | Wgap |  | CC2 |  |  | CC3 |  |  | CC4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB_agg |  | Note1 |  | [MHz] |  | Note1 |  |  | Note1 |  |  | Note1 |  |
|  | [RB] | BW | NUL/DL | fUL/DL |  | BW | NUL/DL | fUL/DL | BW | NUL/DL | fUL/DL | BW | NUL/DL | fUL/DL |
|  |  | [RB] |  | [MHz] |  | [RB] |  | [MHz] | [RB] |  | [MHz] | [RB] |  | [MHz] |
| Max Wgap | 25+50+ 75+100 | 25 | 39675 | 2498.5 | 144.9 | 50 | 41199 | 2650.9 | 75 | 41319 | 2662.9 | 100 | 41490 | 2680 |
|  |  | 25 | 39675 | 2498.5 | 144.7 | 50 | 41197 | 2650.7 | 100 | 41341 | 2665.1 | 75 | 41512 | 2682.2 |
|  |  | 25 | 39675 | 2498.5 | 145.1 | 75 | 41226 | 2653.6 | 50 | 41346 | 2665.6 | 100 | 41490 | 2680 |
|  |  | 25 | 39675 | 2498.5 | 144.5 | 75 | 41220 | 2653 | 100 | 41391 | 2670.1 | 50 | 41535 | 2684.5 |
|  |  | 25 | 39675 | 2498.5 | 144.8 | 100 | 41248 | 2655.8 | 50 | 41392 | 2670.2 | 75 | 41512 | 2682.2 |
|  |  | 25 | 39675 | 2498.5 | 144.4 | 100 | 41244 | 2655.4 | 75 | 41415 | 2672.5 | 50 | 41535 | 2684.5 |
|  | 25+50+ 100+100 | 25 | 39675 | 2498.5 | 139.8 | 50 | 41148 | 2645.8 | 100 | 41292 | 2660.2 | 100 | 41490 | 2680 |
|  |  | 25 | 39675 | 2498.5 | 140.2 | 100 | 41202 | 2651.2 | 50 | 41346 | 2665.6 | 100 | 41490 | 2680 |
|  |  | 25 | 39675 | 2498.5 | 139.3 | 100 | 41193 | 2650.3 | 100 | 41391 | 2670.1 | 50 | 41535 | 2684.5 |
|  | 25+75+ 75+100 | 25 | 39675 | 2498.5 | 139.4 | 75 | 41169 | 2647.9 | 75 | 41319 | 2662.9 | 100 | 41490 | 2680 |
|  |  | 25 | 39675 | 2498.5 | 139.5 | 75 | 41170 | 2648 | 100 | 41341 | 2665.1 | 75 | 41512 | 2682.2 |
|  |  | 25 | 39675 | 2498.5 | 139.1 | 100 | 41191 | 2650.1 | 75 | 41362 | 2667.2 | 75 | 41512 | 2682.2 |
|  | 25+75+ 100+100 | 25 | 39675 | 2498.5 | 134.6 | 75 | 41121 | 2643.1 | 100 | 41292 | 2660.2 | 100 | 41490 | 2680 |
|  |  | 25 | 39675 | 2498.5 | 134.8 | 100 | 41148 | 2645.8 | 75 | 41319 | 2662.9 | 100 | 41490 | 2680 |
|  |  | 25 | 39675 | 2498.5 | 134.3 | 100 | 41143 | 2645.3 | 100 | 41341 | 2665.1 | 75 | 41512 | 2682.2 |
|  | 25+100+ 100+100 | 25 | 39675 | 2498.5 | 129.4 | 100 | 41094 | 2640.4 | 100 | 41292 | 2660.2 | 100 | 41490 | 2680 |
|  | 50+50+ 75+100 | 50 | 39700 | 2501 | 139.9 | 50 | 41199 | 2650.9 | 75 | 41319 | 2662.9 | 100 | 41490 | 2680 |
|  |  | 50 | 39700 | 2501 | 139.7 | 50 | 41197 | 2650.7 | 100 | 41341 | 2665.1 | 75 | 41512 | 2682.2 |
|  |  | 50 | 39700 | 2501 | 140.1 | 75 | 41226 | 2653.6 | 50 | 41346 | 2665.6 | 100 | 41490 | 2680 |
|  |  | 50 | 39700 | 2501 | 139.5 | 75 | 41220 | 2653 | 100 | 41391 | 2670.1 | 50 | 41535 | 2684.5 |
|  |  | 50 | 39700 | 2501 | 139.8 | 100 | 41248 | 2655.8 | 50 | 41392 | 2670.2 | 75 | 41512 | 2682.2 |
|  |  | 50 | 39700 | 2501 | 139.4 | 100 | 41244 | 2655.4 | 75 | 41415 | 2672.5 | 50 | 41535 | 2684.5 |
|  | 50+50+ 100+100 | 50 | 39700 | 2501 | 134.8 | 50 | 41148 | 2645.8 | 100 | 41292 | 2660.2 | 100 | 41490 | 2680 |
|  |  | 50 | 39700 | 2501 | 135.2 | 100 | 41202 | 2651.2 | 50 | 41346 | 2665.6 | 100 | 41490 | 2680 |
|  |  | 50 | 39700 | 2501 | 134.3 | 100 | 41193 | 2650.3 | 100 | 41391 | 2670.1 | 50 | 41535 | 2684.5 |
|  | 50+75+ 75+100 | 50 | 39700 | 2501 | 134.4 | 75 | 41169 | 2647.9 | 75 | 41319 | 2662.9 | 100 | 41490 | 2680 |
|  |  | 50 | 39700 | 2501 | 134.5 | 75 | 41170 | 2648 | 100 | 41341 | 2665.1 | 75 | 41512 | 2682.2 |
|  |  | 50 | 39700 | 2501 | 134.1 | 100 | 41191 | 2650.1 | 75 | 41362 | 2667.2 | 75 | 41512 | 2682.2 |
|  |  | 75 | 39725 | 2503.5 | 134.9 | 50 | 41199 | 2650.9 | 75 | 41319 | 2662.9 | 100 | 41490 | 2680 |
|  |  | 75 | 39725 | 2503.5 | 134.7 | 50 | 41197 | 2650.7 | 100 | 41341 | 2665.1 | 75 | 41512 | 2682.2 |
|  |  | 75 | 39725 | 2503.5 | 135.1 | 75 | 41226 | 2653.6 | 50 | 41346 | 2665.6 | 100 | 41490 | 2680 |
|  |  | 75 | 39725 | 2503.5 | 134.5 | 75 | 41220 | 2653 | 100 | 41391 | 2670.1 | 50 | 41535 | 2684.5 |
|  |  | 75 | 39725 | 2503.5 | 134.8 | 100 | 41248 | 2655.8 | 50 | 41392 | 2670.2 | 75 | 41512 | 2682.2 |
|  |  | 75 | 39725 | 2503.5 | 134.4 | 100 | 41244 | 2655.4 | 75 | 41415 | 2672.5 | 50 | 41535 | 2684.5 |
|  | 50+75+ 100+100 | 50 | 39700 | 2501 | 129.6 | 75 | 41121 | 2643.1 | 100 | 41292 | 2660.2 | 100 | 41490 | 2680 |
|  |  | 50 | 39700 | 2501 | 129.8 | 100 | 41148 | 2645.8 | 75 | 41319 | 2662.9 | 100 | 41490 | 2680 |
|  |  | 50 | 39700 | 2501 | 129.3 | 100 | 41143 | 2645.3 | 100 | 41341 | 2665.1 | 75 | 41512 | 2682.2 |
|  |  | 75 | 39725 | 2503.5 | 129.8 | 50 | 41148 | 2645.8 | 100 | 41292 | 2660.2 | 100 | 41490 | 2680 |
|  |  | 75 | 39725 | 2503.5 | 130.2 | 100 | 41202 | 2651.2 | 50 | 41346 | 2665.6 | 100 | 41490 | 2680 |
|  |  | 75 | 39725 | 2503.5 | 129.3 | 100 | 41193 | 2650.3 | 100 | 41391 | 2670.1 | 50 | 41535 | 2684.5 |
|  |  | 100 | 39750 | 2506 | 129.9 | 50 | 41199 | 2650.9 | 75 | 41319 | 2662.9 | 100 | 41490 | 2680 |
|  |  | 100 | 39750 | 2506 | 129.7 | 50 | 41197 | 2650.7 | 100 | 41341 | 2665.1 | 75 | 41512 | 2682.2 |
|  |  | 100 | 39750 | 2506 | 130.1 | 75 | 41226 | 2653.6 | 50 | 41346 | 2665.6 | 100 | 41490 | 2680 |
|  |  | 100 | 39750 | 2506 | 129.5 | 75 | 41220 | 2653 | 100 | 41391 | 2670.1 | 50 | 41535 | 2684.5 |
|  |  | 100 | 39750 | 2506 | 129.8 | 100 | 41248 | 2655.8 | 50 | 41392 | 2670.2 | 75 | 41512 | 2682.2 |
|  |  | 100 | 39750 | 2506 | 129.4 | 100 | 41244 | 2655.4 | 75 | 41415 | 2672.5 | 50 | 41535 | 2684.5 |
|  | 50+100+ 100+100 | 50 | 39700 | 2501 | 124.4 | 100 | 41094 | 2640.4 | 100 | 41292 | 2660.2 | 100 | 41490 | 2680 |
|  |  | 100 | 39750 | 2506 | 124.8 | 50 | 41148 | 2645.8 | 100 | 41292 | 2660.2 | 100 | 41490 | 2680 |
|  |  | 100 | 39750 | 2506 | 125.2 | 100 | 41202 | 2651.2 | 50 | 41346 | 2665.6 | 100 | 41490 | 2680 |
|  |  | 100 | 39750 | 2506 | 124.3 | 100 | 41193 | 2650.3 | 100 | 41391 | 2670.1 | 50 | 41535 | 2684.5 |
|  | 75+75+ 75+100 | 75 | 39725 | 2503.5 | 129.4 | 75 | 41169 | 2647.9 | 75 | 41319 | 2662.9 | 100 | 41490 | 2680 |
|  |  | 75 | 39725 | 2503.5 | 129.5 | 75 | 41170 | 2648 | 100 | 41341 | 2665.1 | 75 | 41512 | 2682.2 |
|  |  | 75 | 39725 | 2503.5 | 129.1 | 100 | 41191 | 2650.1 | 75 | 41362 | 2667.2 | 75 | 41512 | 2682.2 |
|  | 75+75+ 100+100 | 75 | 39725 | 2503.5 | 124.6 | 75 | 41121 | 2643.1 | 100 | 41292 | 2660.2 | 100 | 41490 | 2680 |
|  |  | 75 | 39725 | 2503.5 | 124.8 | 100 | 41148 | 2645.8 | 75 | 41319 | 2662.9 | 100 | 41490 | 2680 |
|  |  | 75 | 39725 | 2503.5 | 124.3 | 100 | 41143 | 2645.3 | 100 | 41341 | 2665.1 | 75 | 41512 | 2682.2 |
|  |  | 100 | 39750 | 2506 | 124.4 | 75 | 41169 | 2647.9 | 75 | 41319 | 2662.9 | 100 | 41490 | 2680 |
|  |  | 100 | 39750 | 2506 | 124.5 | 75 | 41170 | 2648 | 100 | 41341 | 2665.1 | 75 | 41512 | 2682.2 |
|  |  | 100 | 39750 | 2506 | 124.1 | 100 | 41191 | 2650.1 | 75 | 41362 | 2667.2 | 75 | 41512 | 2682.2 |
|  | 75+100+ 100+100 | 75 | 39725 | 2503.5 | 119.4 | 100 | 41094 | 2640.4 | 100 | 41292 | 2660.2 | 100 | 41490 | 2680 |
|  |  | 100 | 39750 | 2506 | 119.6 | 75 | 41121 | 2643.1 | 100 | 41292 | 2660.2 | 100 | 41490 | 2680 |
|  |  | 100 | 39750 | 2506 | 119.8 | 100 | 41148 | 2645.8 | 75 | 41319 | 2662.9 | 100 | 41490 | 2680 |
|  |  | 100 | 39750 | 2506 | 119.3 | 100 | 41143 | 2645.3 | 100 | 41341 | 2665.1 | 75 | 41512 | 2682.2 |
|  | 100+100+ 100+100 | 100 | 39750 | 2506 | 114.4 | 100 | 41094 | 2640.4 | 100 | 41292 | 2660.2 | 100 | 41490 | 2680 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.9A-8: Test frequencies for CA_41D-41A

| Test frequency ID | CC-Combo / |  | CC1 |  |  | CC2 |  |  | CC3 |  | Wgap |  | CC4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB_agg |  | Note1 |  |  | Note1 |  |  | Note1 |  | [MHz] |  | Note1 |  |
|  | [RB] | BW | NUL/DL | fUL/DL | BW | NUL/DL | fUL/DL | BW | NUL/DL | fUL/DL |  | BW | NUL/DL | fUL/DL |
|  |  | [RB] |  | [MHz] | [RB] |  | [MHz] | [RB] |  | [MHz] |  | [RB] |  | [MHz] |
| Max Wgap | 25+50+ 75+100 | 50 | 39705 | 2501.5 | 75 | 39825 | 2513.5 | 100 | 39996 | 2530.6 | 144.4 | 25 | 41565 | 2687.5 |
|  |  | 50 | 39705 | 2501.5 | 100 | 39849 | 2515.9 | 75 | 40020 | 2533 | 144.5 | 25 | 41565 | 2687.5 |
|  |  | 75 | 39728 | 2503.8 | 50 | 39848 | 2515.8 | 100 | 39992 | 2530.2 | 144.8 | 25 | 41565 | 2687.5 |
|  |  | 75 | 39728 | 2503.8 | 100 | 39899 | 2520.9 | 50 | 40043 | 2535.3 | 144.7 | 25 | 41565 | 2687.5 |
|  |  | 100 | 39750 | 2506 | 50 | 39894 | 2520.4 | 75 | 40014 | 2532.4 | 145.1 | 25 | 41565 | 2687.5 |
|  |  | 100 | 39750 | 2506 | 75 | 39921 | 2523.1 | 50 | 40041 | 2535.1 | 144.9 | 25 | 41565 | 2687.5 |
|  | 25+50+ 100+100 | 50 | 39705 | 2501.5 | 100 | 39849 | 2515.9 | 100 | 40047 | 2535.7 | 139.3 | 25 | 41565 | 2687.5 |
|  |  | 100 | 39750 | 2506 | 50 | 39894 | 2520.4 | 100 | 40038 | 2534.8 | 140.2 | 25 | 41565 | 2687.5 |
|  |  | 100 | 39750 | 2506 | 100 | 39948 | 2525.8 | 50 | 40092 | 2540.2 | 139.8 | 25 | 41565 | 2687.5 |
|  | 25+75+ 75+100 | 75 | 39728 | 2503.8 | 75 | 39878 | 2518.8 | 100 | 40049 | 2535.9 | 139.1 | 25 | 41565 | 2687.5 |
|  |  | 75 | 39728 | 2503.8 | 100 | 39899 | 2520.9 | 75 | 40070 | 2538 | 139.5 | 25 | 41565 | 2687.5 |
|  |  | 100 | 39750 | 2506 | 75 | 39921 | 2523.1 | 75 | 40071 | 2538.1 | 139.4 | 25 | 41565 | 2687.5 |
|  | 25+75+ 100+100 | 75 | 39728 | 2503.8 | 100 | 39899 | 2520.9 | 100 | 40097 | 2540.7 | 134.3 | 25 | 41565 | 2687.5 |
|  |  | 100 | 39750 | 2506 | 75 | 39921 | 2523.1 | 100 | 40092 | 2540.2 | 134.8 | 25 | 41565 | 2687.5 |
|  |  | 100 | 39750 | 2506 | 100 | 39948 | 2525.8 | 75 | 40119 | 2542.9 | 134.6 | 25 | 41565 | 2687.5 |
|  | 25+100+ 100+100 | 100 | 39750 | 2506 | 100 | 39948 | 2525.8 | 100 | 40146 | 2545.6 | 129.4 | 25 | 41565 | 2687.5 |
|  | 50+50+ 75+100 | 50 | 39705 | 2501.5 | 75 | 39825 | 2513.5 | 100 | 39996 | 2530.6 | 139.4 | 50 | 41540 | 2685 |
|  |  | 50 | 39705 | 2501.5 | 100 | 39849 | 2515.9 | 75 | 40020 | 2533 | 139.5 | 50 | 41540 | 2685 |
|  |  | 75 | 39728 | 2503.8 | 50 | 39848 | 2515.8 | 100 | 39992 | 2530.2 | 139.8 | 50 | 41540 | 2685 |
|  |  | 75 | 39728 | 2503.8 | 100 | 39899 | 2520.9 | 50 | 40043 | 2535.3 | 139.7 | 50 | 41540 | 2685 |
|  |  | 100 | 39750 | 2506 | 50 | 39894 | 2520.4 | 75 | 40014 | 2532.4 | 140.1 | 50 | 41540 | 2685 |
|  |  | 100 | 39750 | 2506 | 75 | 39921 | 2523.1 | 50 | 40041 | 2535.1 | 139.9 | 50 | 41540 | 2685 |
|  | 50+50+ 100+100 | 50 | 39705 | 2501.5 | 100 | 39849 | 2515.9 | 100 | 40047 | 2535.7 | 134.3 | 50 | 41540 | 2685 |
|  |  | 100 | 39750 | 2506 | 50 | 39894 | 2520.4 | 100 | 40038 | 2534.8 | 135.2 | 50 | 41540 | 2685 |
|  |  | 100 | 39750 | 2506 | 100 | 39948 | 2525.8 | 50 | 40092 | 2540.2 | 134.8 | 50 | 41540 | 2685 |
|  | 50+75+ 75+100 | 75 | 39728 | 2503.8 | 75 | 39878 | 2518.8 | 100 | 40049 | 2535.9 | 134.1 | 50 | 41540 | 2685 |
|  |  | 75 | 39728 | 2503.8 | 100 | 39899 | 2520.9 | 75 | 40070 | 2538 | 134.5 | 50 | 41540 | 2685 |
|  |  | 100 | 39750 | 2506 | 75 | 39921 | 2523.1 | 75 | 40071 | 2538.1 | 134.4 | 50 | 41540 | 2685 |
|  |  | 50 | 39705 | 2501.5 | 75 | 39825 | 2513.5 | 100 | 39996 | 2530.6 | 134.4 | 75 | 41515 | 2682.5 |
|  |  | 50 | 39705 | 2501.5 | 100 | 39849 | 2515.9 | 75 | 40020 | 2533 | 134.5 | 75 | 41515 | 2682.5 |
|  |  | 75 | 39728 | 2503.8 | 50 | 39848 | 2515.8 | 100 | 39992 | 2530.2 | 134.8 | 75 | 41515 | 2682.5 |
|  |  | 75 | 39728 | 2503.8 | 100 | 39899 | 2520.9 | 50 | 40043 | 2535.3 | 134.7 | 75 | 41515 | 2682.5 |
|  |  | 100 | 39750 | 2506 | 50 | 39894 | 2520.4 | 75 | 40014 | 2532.4 | 135.1 | 75 | 41515 | 2682.5 |
|  |  | 100 | 39750 | 2506 | 75 | 39921 | 2523.1 | 50 | 40041 | 2535.1 | 134.9 | 75 | 41515 | 2682.5 |
|  | 50+75+ 100+100 | 75 | 39728 | 2503.8 | 100 | 39899 | 2520.9 | 100 | 40097 | 2540.7 | 129.3 | 50 | 41540 | 2685 |
|  |  | 100 | 39750 | 2506 | 75 | 39921 | 2523.1 | 100 | 40092 | 2540.2 | 129.8 | 50 | 41540 | 2685 |
|  |  | 100 | 39750 | 2506 | 100 | 39948 | 2525.8 | 75 | 40119 | 2542.9 | 129.6 | 50 | 41540 | 2685 |
|  |  | 50 | 39705 | 2501.5 | 100 | 39849 | 2515.9 | 100 | 40047 | 2535.7 | 129.3 | 75 | 41515 | 2682.5 |
|  |  | 100 | 39750 | 2506 | 50 | 39894 | 2520.4 | 100 | 40038 | 2534.8 | 130.2 | 75 | 41515 | 2682.5 |
|  |  | 100 | 39750 | 2506 | 100 | 39948 | 2525.8 | 50 | 40092 | 2540.2 | 129.8 | 75 | 41515 | 2682.5 |
|  |  | 50 | 39705 | 2501.5 | 75 | 39825 | 2513.5 | 100 | 39996 | 2530.6 | 134.4 | 100 | 41490 | 2680 |
|  |  | 50 | 39705 | 2501.5 | 100 | 39849 | 2515.9 | 75 | 40020 | 2533 | 129.5 | 100 | 41490 | 2680 |
|  |  | 75 | 39728 | 2503.8 | 50 | 39848 | 2515.8 | 100 | 39992 | 2530.2 | 129.8 | 100 | 41490 | 2680 |
|  |  | 75 | 39728 | 2503.8 | 100 | 39899 | 2520.9 | 50 | 40043 | 2535.3 | 129.7 | 100 | 41490 | 2680 |
|  |  | 100 | 39750 | 2506 | 50 | 39894 | 2520.4 | 75 | 40014 | 2532.4 | 130.1 | 100 | 41490 | 2680 |
|  |  | 100 | 39750 | 2506 | 75 | 39921 | 2523.1 | 50 | 40041 | 2535.1 | 129.9 | 100 | 41490 | 2680 |
|  | 50+100+ 100+100 | 100 | 39750 | 2506 | 100 | 39948 | 2525.8 | 100 | 40146 | 2545.6 | 124.4 | 50 | 41540 | 2685 |
|  |  | 50 | 39705 | 2501.5 | 100 | 39849 | 2515.9 | 100 | 40047 | 2535.7 | 124.3 | 100 | 41490 | 2680 |
|  |  | 100 | 39750 | 2506 | 50 | 39894 | 2520.4 | 100 | 40038 | 2534.8 | 125.2 | 100 | 41490 | 2680 |
|  |  | 100 | 39750 | 2506 | 100 | 39948 | 2525.8 | 50 | 40092 | 2540.2 | 124.8 | 100 | 41490 | 2680 |
|  | 75+75+ 75+100 | 75 | 39728 | 2503.8 | 75 | 39878 | 2518.8 | 100 | 40049 | 2535.9 | 129.1 | 75 | 41515 | 2682.5 |
|  |  | 75 | 39728 | 2503.8 | 100 | 39899 | 2520.9 | 75 | 40070 | 2538 | 129.5 | 75 | 41515 | 2682.5 |
|  |  | 100 | 39750 | 2506 | 75 | 39921 | 2523.1 | 75 | 40071 | 2538.1 | 129.4 | 75 | 41515 | 2682.5 |
|  | 75+75+ 100+100 | 75 | 39728 | 2503.8 | 100 | 39899 | 2520.9 | 100 | 40097 | 2540.7 | 124.3 | 75 | 41515 | 2682.5 |
|  |  | 100 | 39750 | 2506 | 75 | 39921 | 2523.1 | 100 | 40092 | 2540.2 | 124.8 | 75 | 41515 | 2682.5 |
|  |  | 100 | 39750 | 2506 | 100 | 39948 | 2525.8 | 75 | 40119 | 2542.9 | 124.6 | 75 | 41515 | 2682.5 |
|  |  | 75 | 39728 | 2503.8 | 75 | 39878 | 2518.8 | 100 | 40049 | 2535.9 | 124.1 | 100 | 41490 | 2680 |
|  |  | 75 | 39728 | 2503.8 | 100 | 39899 | 2520.9 | 75 | 40070 | 2538 | 124.5 | 100 | 41490 | 2680 |
|  |  | 100 | 39750 | 2506 | 75 | 39921 | 2523.1 | 75 | 40071 | 2538.1 | 124.4 | 100 | 41490 | 2680 |
|  | 75+100+ 100+100 | 100 | 39750 | 2506 | 100 | 39948 | 2525.8 | 100 | 40146 | 2545.6 | 119.4 | 75 | 41515 | 2682.5 |
|  |  | 75 | 39728 | 2503.8 | 100 | 39899 | 2520.9 | 100 | 40097 | 2540.7 | 119.3 | 100 | 41490 | 2680 |
|  |  | 100 | 39750 | 2506 | 75 | 39921 | 2523.1 | 100 | 40092 | 2540.2 | 119.8 | 100 | 41490 | 2680 |
|  |  | 100 | 39750 | 2506 | 100 | 39948 | 2525.8 | 75 | 40119 | 2542.9 | 119.6 | 100 | 41490 | 2680 |
|  | 100+100+ 100+100 | 100 | 39750 | 2506 | 100 | 39948 | 2525.8 | 100 | 40146 | 2545.6 | 114.4 | 100 | 41490 | 2680 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.9A-9: Test frequencies for CA_41E

| Range | CC-Combo / NRB_agg [RB] |  | CC1 Note1 |  |  | CC2 Note1 |  |  | CC3 Note1 |  |  |  |  | CC4 Note1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL/DL | fUL/DL [MHz] | BW [RB] | NUL/DL | fUL/DL [MHz] | BW [RB] | NUL/DL | fUL/DL [MHz] | BW [RB] |  |  | NUL/DL |  |  | fUL/DL [MHz] |  |  |
| Low | 75+75+75+100 | 75 | 39725 | 2503.5 | 75 | 39875 | 2518.5 | 75 | 40025 | 2533.5 | 100 |  |  | 40200 |  |  | 2551.0 |  |  |
|  | 75+75+100+100 | 75 | 39725 | 2503.5 | 75 | 39725 | 2518.5 | 100 | 40050 | 2536.0 | 100 |  |  | 40250 |  |  | 2556.0 |  |  |
|  |  | 75 | 39725 | 2503.5 | 100 | 39900 | 2521.0 | 75 | 40075 | 2538.5 | 100 |  |  | 40250 |  |  | 2556.0 |  |  |
|  |  | 100 | 39750 | 2506.0 | 75 | 39925 | 2523.5 | 75 | 40075 | 2538.5 | 100 |  |  | 40250 |  |  | 2556.0 |  |  |
|  | 75+100+100+100 | 75 | 39725 | 2503.5 | 100 | 39900 | 2521.0 | 100 | 40100 | 2541.0 | 100 |  |  | 40300 |  |  | 2561.0 |  |  |
|  |  | 100 | 39750 | 2506.0 | 75 | 39925 | 2523.5 | 100 | 40100 | 2541.0 | 100 |  |  | 40300 |  |  | 2561.0 |  |  |
|  |  | 100 | 39750 | 2506.0 | 100 | 39950 | 2526.0 | 75 | 40125 | 2543.5 | 100 |  |  | 40300 |  |  | 2561.0 |  |  |
|  | 100+100+100+100 | 100 | 39750 | 2506.0 | 100 | 39950 | 2526.0 | 100 | 40150 | 2546.0 | 100 |  |  | 40350 |  |  | 2566.0 |  |  |
| Mid | 75+75+75+100 | 75 | 40370 | 2568.0 | 75 | 40520 | 2583.0 | 75 | 40670 | 2598.0 | 100 |  |  | 40845 |  |  | 2615.5 |  |  |
|  | 75+75+100+100 | 75 | 40345 | 2565.5 | 75 | 40495 | 2580.5 | 100 | 40670 | 2598.0 | 100 |  |  | 40870 |  |  | 2618.0 |  |  |
|  |  | 75 | 40345 | 2565.5 | 100 | 40520 | 2583.0 | 75 | 40695 | 2600.5 | 100 |  |  | 40870 |  |  | 2618.0 |  |  |
|  |  | 100 | 40370 | 2568.0 | 75 | 40545 | 2585.5 | 75 | 40695 | 2600.5 | 100 |  |  | 40870 |  |  | 2618.0 |  |  |
|  | 75+100+100+100 | 75 | 40320 | 2563.0 | 100 | 40495 | 2580.5 | 100 | 40695 | 2600.5 | 100 |  |  | 40895 |  |  | 2620.5 |  |  |
|  |  | 100 | 40345 | 2565.5 | 75 | 40520 | 2583.0 | 100 | 40695 | 2600.5 | 100 |  |  | 40895 |  |  | 2620.5 |  |  |
|  |  | 100 | 40345 | 2565.5 | 100 | 40545 | 2585.5 | 75 | 40720 | 2603.0 | 100 |  |  | 40895 |  |  | 2620.5 |  |  |
|  | 100+100+100+100 | 100 | 40320 | 2563.0 | 100 | 40520 | 2583.0 | 100 | 40720 | 2603.0 | 100 |  |  | 40920 |  |  | 2623.0 |  |  |
| High | 75+75+75+100 | 75 | 41015 | 2632.5 | 75 | 41165 | 2647.5 | 75 | 41315 | 2662.5 | 100 |  |  | 41490 |  |  | 2680.0 |  |  |
|  | 75+75+100+100 | 75 | 40965 | 2627.5 | 75 | 41115 | 2642.5 | 100 | 41290 | 2660.0 | 100 |  |  | 41490 |  |  | 2680.0 |  |  |
|  |  | 75 | 40965 | 2627.5 | 100 | 41140 | 2645.0 | 75 | 41315 | 2662.5 | 100 |  |  | 41490 |  |  | 2680.0 |  |  |
|  |  | 100 | 40990 | 2630.0 | 75 | 41165 | 2647.5 | 75 | 41315 | 2662.5 | 100 |  |  | 41490 |  |  | 2680.0 |  |  |
|  | 75+100+100+100 | 75 | 40915 | 2622.5 | 100 | 41090 | 2640.0 | 100 | 41290 | 2660.0 | 100 |  |  | 41490 |  |  | 2680.0 |  |  |
|  |  | 100 | 40940 | 2625.0 | 75 | 41115 | 2642.5 | 100 | 41290 | 2660.0 | 100 |  |  | 41490 |  |  | 2680.0 |  |  |
|  |  | 100 | 40940 | 2625.0 | 100 | 41140 | 2645.0 | 75 | 41315 | 2662.5 | 100 |  |  | 41490 |  |  | 2680.0 |  |  |
|  | 100+100+100+100 | 100 | 40890 | 2620.0 | 100 | 41090 | 2640.0 | 100 | 41290 | 2660.0 | 100 |  |  | 41490 |  |  | 2680.0 |  |  |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.9A-10: Test frequencies for CA_41F

| Range | CC-Combo / NRB_agg [RB] |  | CC1 Note1 |  |  | CC2 Note1 |  |  | CC3 Note1 |  |  | CC4 Note1 |  |  |  |  | CC5 Note1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL/DL | fUL/DL [MHz] | BW [RB] | NUL/DL | fUL/DL [MHz] | BW [RB] | NUL/DL | fUL/DL [MHz] | BW [RB] | NUL/DL | fUL/DL [MHz] | BW [RB] |  |  | NUL/DL |  |  | fUL/DL [MHz] |  |  |
| Low | 50+75+100+100+100 | 50 | 39705 | 2501.5 | 75 | 39825 | 2513.5 | 100 | 39996 | 2530.6 | 100 | 40194 | 2550.4 | 100 |  |  | 40392 |  |  | 2570.2 |  |  |
|  | 50+100+100+100+100 | 50 | 39705 | 2501.5 | 100 | 39849 | 2515.9 | 100 | 40047 | 2535.7 | 100 | 40245 | 2555.5 | 100 |  |  | 40443 |  |  | 2575.3 |  |  |
|  | 75+75+100+100+100 | 75 | 39728 | 2503.8 | 75 | 39878 | 2518.8 | 100 | 40049 | 2535.9 | 100 | 40247 | 2555.7 | 100 |  |  | 40445 |  |  | 2575.5 |  |  |
|  | 75+100+100+100+1100 | 75 | 39728 | 2503.8 | 100 | 39899 | 2520.9 | 100 | 40097 | 2540.7 | 100 | 40295 | 2560.5 | 100 |  |  | 40493 |  |  | 2580.3 |  |  |
|  |  | 100 | 39750 | 2506 | 75 | 39921 | 2523.1 | 100 | 40092 | 2540.2 | 100 | 40290 | 2560 | 100 |  |  | 40488 |  |  | 2579.8 |  |  |
|  | 100+100+100+100+100 | 100 | 39750 | 2506 | 100 | 39948 | 2525.8 | 100 | 40146 | 2545.6 | 100 | 40344 | 2565.4 | 100 |  |  | 40542 |  |  | 2585.2 |  |  |
| Mid | 50+75+100+100+100 | 50 | 40254 | 2556.4 | 75 | 40374 | 2568.4 | 100 | 40545 | 2585.5 | 100 | 40743 | 2605.3 | 100 |  |  | 40941 |  |  | 2625.1 |  |  |
|  | 50+100+100+100+100 | 50 | 40229 | 2553.9 | 100 | 40373 | 2568.3 | 100 | 40571 | 2588.1 | 100 | 40769 | 2607.9 | 100 |  |  | 40967 |  |  | 2627.7 |  |  |
|  | 75+75+100+100+100 | 75 | 40250 | 2556 | 75 | 40400 | 2571 | 100 | 40571 | 2588.1 | 100 | 40769 | 2607.9 | 100 |  |  | 40967 |  |  | 2627.7 |  |  |
|  | 75+100+100+100+1100 | 75 | 40226 | 2553.6 | 100 | 40397 | 2570.7 | 100 | 40595 | 2590.5 | 100 | 40793 | 2610.3 | 100 |  |  | 40991 |  |  | 2630.1 |  |  |
|  |  | 100 | 40251 | 2556.1 | 75 | 40422 | 2573.2 | 100 | 40593 | 2590.3 | 100 | 40791 | 2610.1 | 100 |  |  | 40989 |  |  | 2629.9 |  |  |
|  | 100+100+100+100+100 | 100 | 40224 | 2553.4 | 100 | 40422 | 2573.2 | 100 | 40620 | 2593 | 100 | 40818 | 2612.8 | 100 |  |  | 41016 |  |  | 2632.6 |  |  |
| High | 50+75+100+100+100 | 50 | 40803 | 2611.3 | 75 | 40923 | 2623.3 | 100 | 41094 | 2640.4 | 100 | 41292 | 2660.2 | 100 |  |  | 41490 |  |  | 2680 |  |  |
|  | 50+100+100+100+100 | 50 | 40752 | 2606.2 | 100 | 40896 | 2620.6 | 100 | 41094 | 2640.4 | 100 | 41292 | 2660.2 | 100 |  |  | 41490 |  |  | 2680 |  |  |
|  | 75+75+100+100+100 | 75 | 40773 | 2608.3 | 75 | 40923 | 2623.3 | 100 | 41094 | 2640.4 | 100 | 41292 | 2660.2 | 100 |  |  | 41490 |  |  | 2680 |  |  |
|  | 75+100+100+100+1100 | 75 | 40725 | 2603.5 | 100 | 40896 | 2620.6 | 100 | 41094 | 2640.4 | 100 | 41292 | 2660.2 | 100 |  |  | 41490 |  |  | 2680 |  |  |
|  |  | 100 | 40752 | 2606.2 | 75 | 40923 | 2623.3 | 100 | 41094 | 2640.4 | 100 | 41292 | 2660.2 | 100 |  |  | 41490 |  |  | 2680 |  |  |
|  | 100+100+100+100+100 | 100 | 40698 | 2600.8 | 100 | 40896 | 2620.6 | 100 | 41094 | 2640.4 | 100 | 41292 | 2660.2 | 100 |  |  | 41490 |  |  | 2680 |  |  |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### 4.3.1.2.10	TDD reference test frequencies for Operating Band 42

Table 4.3.1.2.10-1: Test frequencies for E-UTRA channel bandwidth for operating band 42

| Test Frequency ID | Bandwidth [MHz] | EARFCN | Frequency (UL and DL) [MHz] |
| --- | --- | --- | --- |
| Low Range | 5 | 41615 | 3402.5 |
|  | 10 | 41640 | 3405 |
|  | 15 | 41665 | 3407.5 |
|  | 20 | 41690 | 3410 |
| Mid Range | 5/10/15/20 | 42590 | 3500 |
| High Range | 5 | 43565 | 3597.5 |
|  | 10 | 43540 | 3595 |
|  | 15 | 43515 | 3592.5 |
|  | 20 | 43490 | 3590 |

##### 4.3.1.2.10A	TDD reference test frequencies for CA in operating band 42

Table 4.3.1.2.10A-1: Test frequencies for CA_42C

| Range | CC-Combo / NRB_agg [RB] |  | CC1 Note1 |  |  | CC2 Note1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL/DL | fUL/DL [MHz] | BW [RB] | NUL/DL | fUL/DL [MHz] |
| Low | 25+100 | 25 | 41623 | 3403.3 | 100 | 41740 | 3415 |
|  |  | 100 | 41690 | 3410 | 25 | 41807 | 3421.7 |
|  | 50+100 | 50 | 41645 | 3405.5 | 100 | 41789 | 3419.9 |
|  |  | 100 | 41690 | 3410 | 50 | 41834 | 3424.4 |
|  | 75+100 | 75 | 41668 | 3407.8 | 100 | 41839 | 3424.9 |
|  |  | 100 | 41690 | 3410 | 75 | 41861 | 3427.1 |
|  | 100+100 | 100 | 41690 | 3410 | 100 | 41888 | 3429.8 |
| Mid | 25+100 | 25 | 42498 | 3490.8 | 100 | 42615 | 3502.5 |
|  |  | 100 | 42565 | 3497.5 | 25 | 42682 | 3509.2 |
|  | 50+100 | 50 | 42496 | 3490.6 | 100 | 42640 | 3505 |
|  |  | 100 | 42541 | 3495.1 | 50 | 42685 | 3509.5 |
|  | 75+100 | 75 | 42493 | 3490.3 | 100 | 42664 | 3507.4 |
|  |  | 100 | 42516 | 3492.6 | 75 | 42687 | 3509.7 |
|  | 100+100 | 100 | 42491 | 3490.1 | 100 | 42689 | 3509.9 |
| High | 25+100 | 25 | 43373 | 3578.3 | 100 | 43490 | 3590 |
|  |  | 100 | 43440 | 3585 | 25 | 43557 | 3596.7 |
|  | 50+100 | 50 | 43346 | 3575.6 | 100 | 43490 | 3590 |
|  |  | 100 | 43391 | 3580.1 | 50 | 43535 | 3594.5 |
|  | 75+100 | 75 | 43319 | 3572.9 | 100 | 43490 | 3590 |
|  |  | 100 | 43341 | 3575.1 | 75 | 43512 | 3592.2 |
|  | 100+100 | 100 | 43292 | 3570.2 | 100 | 43490 | 3590 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |

Table 4.3.1.2.10A-2: Test frequencies for CA_42A-42A

| Test Frequency ID | CC-Combo / NRB_agg [RB] |  | CC1 Note1 |  | Wgap [MHz] |  | CC2 Note1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL/DL | fUL/DL [MHz] |  | BW [RB] | NUL/DL | fUL/DL [MHz] |
| Max WGap | 25+25 | 25 | 41615 | 3402.5 | 190 | 25 | 43565 | 3597.5 |
|  | 25+50 | 25 | 41615 | 3402.5 | 185 | 50 | 43540 | 3595 |
|  |  | 50 | 41640 | 3405 | 185 | 25 | 43565 | 3597.5 |
|  | 25+75 | 25 | 41615 | 3402.5 | 180 | 75 | 43515 | 3592.5 |
|  |  | 75 | 41665 | 3407.5 | 180 | 25 | 43565 | 3597.5 |
|  | 50+50 | 50 | 41640 | 3405 | 180 | 50 | 43540 | 3595 |
|  | 25+100 | 25 | 41615 | 3402.5 | 175 | 100 | 43490 | 3590 |
|  |  | 100 | 41690 | 3410 | 175 | 25 | 43565 | 3597.5 |
|  | 50+75 | 50 | 41640 | 3405 | 175 | 75 | 43515 | 3592.5 |
|  |  | 75 | 41665 | 3407.5 | 175 | 50 | 43540 | 3595 |
|  | 50+100 | 50 | 41640 | 3405 | 170 | 100 | 43490 | 3590 |
|  |  | 100 | 41690 | 3410 | 170 | 50 | 43540 | 3595 |
|  | 75+100 | 75 | 41665 | 3407.5 | 165 | 100 | 43490 | 3590 |
|  |  | 100 | 41690 | 3410 | 165 | 75 | 43515 | 3592.5 |
|  | 100+100 | 100 | 41690 | 3410 | 160 | 100 | 43490 | 3590 |
| Refsens2 | 100+100 | 100 | 42590 | 3500 | 70 | 100 | 43490 | 3590 |
| Note 1: Carriers in increasing frequency order. Note 2: Test point derived with regard to REFSENS requirements. |  |  |  |  |  |  |  |  |

Table 4.3.1.2.10A-3: Test frequencies for CA_42D

| Range | CC-Combo / NRB_agg [RB] |  | CC1 Note1 |  |  | CC2 Note1 |  | CC3 Note1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL/DL | fUL/DL [MHz] | BW [RB] | NUL/DL | fUL/DL [MHz] | BW [RB] | NUL/DL | fUL/DL [MHz] |
| Low | 25+100+100 | 25 | 41623 | 3403.3 | 100 | 41740 | 3415 | 100 | 41938 | 3434.8 |
|  |  | 100 | 41690 | 3410 | 100 | 41888 | 3429.8 | 25 | 42005 | 3441.5 |
|  | 50+100+100 | 50 | 41645 | 3405.5 | 100 | 41789 | 3419.9 | 100 | 41987 | 3439.7 |
|  |  | 100 | 41690 | 3410 | 100 | 41888 | 3429.8 | 50 | 42032 | 3444.2 |
|  | 75+100+100 | 75 | 41668 | 3407.8 | 100 | 41839 | 3424.9 | 100 | 42037 | 3444.7 |
|  |  | 100 | 41690 | 3410 | 100 | 41888 | 3429.8 | 75 | 42059 | 3446.9 |
|  | 100+100+100 | 100 | 41690 | 3410 | 100 | 41888 | 3429.8 | 100 | 42086 | 3449.6 |
| Mid | 25+100+100 | 25 | 42399 | 3480.9 | 100 | 42516 | 3492.6 | 100 | 42714 | 3512.4 |
|  |  | 100 | 42466 | 3487.6 | 100 | 42664 | 3507.4 | 25 | 42781 | 3519.1 |
|  | 50+100+100 | 50 | 42397 | 3480.7 | 100 | 42541 | 3495.1 | 100 | 42739 | 3514.9 |
|  |  | 100 | 42442 | 3485.2 | 100 | 42640 | 3505 | 50 | 42784 | 3519.4 |
|  | 75+100+100 | 75 | 42394 | 3480.4 | 100 | 42565 | 3497.5 | 100 | 42763 | 3517.3 |
|  |  | 100 | 42417 | 3482.7 | 100 | 42615 | 3502.5 | 75 | 42786 | 3519.6 |
|  | 100+100+100 | 100 | 42392 | 3480.2 | 100 | 42590 | 3500 | 100 | 42788 | 3519.8 |
| High | 25+100+100 | 25 | 43175 | 3558.5 | 100 | 43292 | 3570.2 | 100 | 43490 | 3590 |
|  |  | 100 | 43242 | 3565.2 | 100 | 43440 | 3585 | 25 | 43557 | 3596.7 |
|  | 50+100+100 | 50 | 43148 | 3555.8 | 100 | 43292 | 3570.2 | 100 | 43490 | 3590 |
|  |  | 100 | 43193 | 3560.3 | 100 | 43391 | 3580.1 | 50 | 43535 | 3594.5 |
|  | 75+100+100 | 75 | 43121 | 3553.1 | 100 | 43292 | 3570.2 | 100 | 43490 | 3590 |
|  |  | 100 | 43143 | 3555.3 | 100 | 43341 | 3575.1 | 75 | 43512 | 3592.2 |
|  | 100+100+100 | 100 | 43094 | 3550.4 | 100 | 43292 | 3570.2 | 100 | 43490 | 3590 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.10A-4: Test frequencies for CA_42C-42A

| Test Frequency ID | CC-Combo / NRB_agg [RB] |  | CC1 Note1 |  |  | CC2 Note1 |  | Wgap [MHz] |  | CC3 Note1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL/DL | fUL/DL [MHz] | BW [RB | NUL/DL | fUL/DL [MHz] |  | BW [RB] | NUL/DL | fUL/DL [MHz] |
| Max WGap | 25+25+100 | 25 | 41623 | 3403.3 | 100 | 41740 | 3415 | 170 | 25 | 43565 | 3597.5 |
|  |  | 100 | 41690 | 3410 | 25 | 41807 | 3421.7 | 170.8 | 25 | 43565 | 3597.5 |
|  | 25+50+100 | 25 | 41623 | 3403.3 | 100 | 41740 | 3415 | 165 | 50 | 43540 | 3595 |
|  |  | 50 | 41645 | 3405.5 | 100 | 41789 | 3419.9 | 165.1 | 25 | 43565 | 3597.5 |
|  |  | 100 | 41690 | 3410 | 25 | 41807 | 3421.7 | 165.8 | 50 | 43540 | 3595 |
|  |  | 100 | 41690 | 3410 | 50 | 41834 | 3424.4 | 165.6 | 25 | 43565 | 3597.5 |
|  | 25+75+100 | 25 | 41623 | 3403.3 | 100 | 41740 | 3415 | 160 | 75 | 43515 | 3592.5 |
|  |  | 75 | 41668 | 3407.8 | 100 | 41839 | 3424.9 | 160.1 | 25 | 43565 | 3597.5 |
|  |  | 100 | 41690 | 3410 | 25 | 41807 | 3421.7 | 160.8 | 75 | 43515 | 3592.5 |
|  |  | 100 | 41690 | 3410 | 75 | 41861 | 3427.1 | 160.4 | 25 | 43565 | 3597.5 |
|  | 50+50+100 | 50 | 41645 | 3405.5 | 100 | 41789 | 3419.9 | 160.1 | 50 | 43540 | 3595 |
|  |  | 100 | 41690 | 3410 | 50 | 41834 | 3424.4 | 160.6 | 50 | 43540 | 3595 |
|  | 25+100+100 | 25 | 41623 | 3403.3 | 100 | 41740 | 3415 | 155 | 100 | 43490 | 3590 |
|  |  | 100 | 41690 | 3410 | 25 | 41807 | 3421.7 | 155.8 | 100 | 43490 | 3590 |
|  |  | 100 | 41690 | 3410 | 100 | 41888 | 3429.8 | 155.2 | 25 | 43565 | 3597.5 |
|  | 50+75+100 | 50 | 41645 | 3405.5 | 100 | 41789 | 3419.9 | 155.1 | 75 | 43515 | 3592.5 |
|  |  | 75 | 41668 | 3407.8 | 100 | 41839 | 3424.9 | 155.1 | 50 | 43540 | 3595 |
|  |  | 100 | 41690 | 3410 | 50 | 41834 | 3424.4 | 155.6 | 75 | 43515 | 3592.5 |
|  |  | 100 | 41690 | 3410 | 75 | 41861 | 3427.1 | 155.4 | 50 | 43540 | 3595 |
|  | 50+100+100 | 50 | 41645 | 3405.5 | 100 | 41789 | 3419.9 | 150.1 | 100 | 43490 | 3590 |
|  |  | 100 | 41690 | 3410 | 50 | 41834 | 3424.4 | 150.6 | 100 | 43490 | 3590 |
|  |  | 100 | 41690 | 3410 | 100 | 41888 | 3429.8 | 150.2 | 50 | 43540 | 3595 |
|  | 75+75+100 | 75 | 41668 | 3407.8 | 100 | 41839 | 3424.9 | 150.1 | 75 | 43515 | 3592.5 |
|  |  | 100 | 41690 | 3410 | 75 | 41861 | 3427.1 | 150.4 | 75 | 43515 | 3592.5 |
|  | 75+100+100 | 75 | 41668 | 3407.8 | 100 | 41839 | 3424.9 | 145.1 | 100 | 43490 | 3590 |
|  |  | 100 | 41690 | 3410 | 75 | 41861 | 3427.1 | 145.4 | 100 | 43490 | 3590 |
|  |  | 100 | 41690 | 3410 | 100 | 41888 | 3429.8 | 145.2 | 75 | 43515 | 3592.5 |
|  | 100+100+100 | 100 | 41690 | 3410 | 100 | 41888 | 3429.8 | 140.2 | 100 | 43490 | 3590 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.10A-5: Test frequencies for CA_42A-42C

| Test Frequency ID | CC-Combo / NRB_agg [RB] |  | CC1 Note1 |  | Wgap [MHz] |  | CC2 Note1 |  |  | CC3 Note1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL/DL | fUL/DL [MHz] |  | BW [RB | NUL/DL | fUL/DL [MHz] | BW [RB] | NUL/DL | fUL/DL [MHz] |
| Max WGap | 25+25+100 | 25 | 41615 | 3402.5 | 170.8 | 25 | 43373 | 3578.3 | 100 | 43490 | 3590 |
|  |  | 25 | 41615 | 3402.5 | 170 | 100 | 43440 | 3585 | 25 | 43557 | 3596.7 |
|  | 25+50+100 | 25 | 41615 | 3402.5 | 165.6 | 50 | 43346 | 3575.6 | 100 | 43490 | 3590 |
|  |  | 25 | 41615 | 3402.5 | 165.1 | 100 | 43391 | 3580.1 | 50 | 43535 | 3594.5 |
|  |  | 50 | 41640 | 3405 | 165.8 | 25 | 43373 | 3578.3 | 100 | 43490 | 3590 |
|  |  | 50 | 41640 | 3405 | 165 | 100 | 43440 | 3585 | 25 | 43557 | 3596.7 |
|  | 25+75+100 | 25 | 41615 | 3402.5 | 160.4 | 75 | 43319 | 3572.9 | 100 | 43490 | 3590 |
|  |  | 25 | 41615 | 3402.5 | 160.1 | 100 | 43341 | 3575.1 | 75 | 43512 | 3592.2 |
|  |  | 75 | 41665 | 3407.5 | 160.8 | 25 | 43373 | 3578.3 | 100 | 43490 | 3590 |
|  |  | 75 | 41665 | 3407.5 | 160 | 100 | 43440 | 3585 | 25 | 43557 | 3596.7 |
|  | 50+50+100 | 50 | 41640 | 3405 | 160.6 | 50 | 43346 | 3575.6 | 100 | 43490 | 3590 |
|  |  | 50 | 41640 | 3405 | 160.1 | 100 | 43391 | 3580.1 | 50 | 43535 | 3594.5 |
|  | 25+100+100 | 25 | 41615 | 3402.5 | 155.2 | 100 | 43292 | 3570.2 | 100 | 43490 | 3590 |
|  |  | 100 | 41690 | 3410 | 155.8 | 25 | 43373 | 3578.3 | 100 | 43490 | 3590 |
|  |  | 100 | 41690 | 3410 | 155 | 100 | 43440 | 3585 | 25 | 43557 | 3596.7 |
|  | 50+75+100 | 50 | 41640 | 3405 | 155.4 | 75 | 43319 | 3572.9 | 100 | 43490 | 3590 |
|  |  | 50 | 41640 | 3405 | 155.1 | 100 | 43341 | 3575.1 | 75 | 43512 | 3592.2 |
|  |  | 75 | 41665 | 3407.5 | 155.6 | 50 | 43346 | 3575.6 | 100 | 43490 | 3590 |
|  |  | 75 | 41665 | 3407.5 | 155.1 | 100 | 43391 | 3580.1 | 50 | 43535 | 3594.5 |
|  | 50+100+100 | 50 | 41640 | 3405 | 150.2 | 100 | 43292 | 3570.2 | 100 | 43490 | 3590 |
|  |  | 100 | 41690 | 3410 | 150.6 | 50 | 43346 | 3575.6 | 100 | 43490 | 3590 |
|  |  | 100 | 41690 | 3410 | 150.1 | 100 | 43391 | 3580.1 | 50 | 43535 | 3594.5 |
|  | 75+75+100 | 75 | 41665 | 3407.5 | 150.4 | 75 | 43319 | 3572.9 | 100 | 43490 | 3590 |
|  |  | 75 | 41665 | 3407.5 | 150.1 | 100 | 43341 | 3575.1 | 75 | 43512 | 3592.2 |
|  | 75+100+100 | 75 | 41665 | 3407.5 | 145.2 | 100 | 43292 | 3570.2 | 100 | 43490 | 3590 |
|  |  | 100 | 41690 | 3410 | 145.4 | 75 | 43319 | 3572.9 | 100 | 43490 | 3590 |
|  |  | 100 | 41690 | 3410 | 145.1 | 100 | 43341 | 3575.1 | 75 | 43512 | 3592.2 |
|  | 100+100+100 | 100 | 41690 | 3410 | 140.2 | 100 | 43292 | 3570.2 | 100 | 43490 | 3590 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.10A-6: Test frequencies for CA_42C-42C

| Test frequency ID | CC-Combo / |  | CC1 |  |  | CC2 |  | Wgap |  | CC3 |  |  | CC4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB_agg |  | Note1 |  |  | Note1 |  | [MHz] |  | Note1 |  |  | Note1 |  |
|  | [RB] | BW | NUL/DL | fUL/DL | BW | NUL/DL | fUL/DL |  | BW | NUL/DL | fUL/DL | BW | NUL/DL | fUL/DL |
|  |  | [RB] |  | [MHz] | [RB] |  | [MHz] |  | [RB] |  | [MHz] | [RB] |  | [MHz] |
| Max Wgap | 25+25+ 100+100 | 25 | 41623 | 3403.3 | 100 | 41740 | 3415 | 150.8 | 25 | 43373 | 3578.3 | 100 | 43490 | 3590 |
|  |  | 100 | 41690 | 3410 | 25 | 41807 | 3421.7 | 150.8 | 100 | 43440 | 3585 | 25 | 43557 | 3596.7 |
|  |  | 25 | 41623 | 3403.3 | 100 | 41740 | 3415 | 150 | 100 | 43440 | 3585 | 25 | 43557 | 3596.7 |
|  |  | 100 | 41690 | 3410 | 25 | 41807 | 3421.7 | 151.6 | 25 | 43373 | 3578.3 | 100 | 43490 | 3590 |
|  | 25+50+ 100+100 | 25 | 41623 | 3403.3 | 100 | 41740 | 3415 | 145.6 | 50 | 43346 | 3575.6 | 100 | 43490 | 3590 |
|  |  | 100 | 41690 | 3410 | 25 | 41807 | 3421.7 | 145.9 | 100 | 43391 | 3580.1 | 50 | 43535 | 3594.5 |
|  |  | 25 | 41623 | 3403.3 | 100 | 41740 | 3415 | 145.1 | 100 | 43391 | 3580.1 | 50 | 43535 | 3594.5 |
|  |  | 100 | 41690 | 3410 | 25 | 41807 | 3421.7 | 146.4 | 50 | 43346 | 3575.6 | 100 | 43490 | 3590 |
|  |  | 50 | 41645 | 3405.5 | 100 | 41789 | 3419.9 | 145.9 | 25 | 43373 | 3578.3 | 100 | 43490 | 3590 |
|  |  | 100 | 41690 | 3410 | 50 | 41834 | 3424.4 | 145.6 | 100 | 43440 | 3585 | 25 | 43557 | 3596.7 |
|  |  | 50 | 41645 | 3405.5 | 100 | 41789 | 3419.9 | 145.1 | 100 | 43440 | 3585 | 25 | 43557 | 3596.7 |
|  |  | 100 | 41690 | 3410 | 50 | 41834 | 3424.4 | 146.4 | 25 | 43373 | 3578.3 | 100 | 43490 | 3590 |
|  | 25+75+ 100+100 | 25 | 41623 | 3403.3 | 100 | 41740 | 3415 | 140.4 | 75 | 43319 | 3572.9 | 100 | 43490 | 3590 |
|  |  | 100 | 41690 | 3410 | 25 | 41807 | 3421.7 | 140.9 | 100 | 43341 | 3575.1 | 75 | 43512 | 3592.2 |
|  |  | 25 | 41623 | 3403.3 | 100 | 41740 | 3415 | 140.1 | 100 | 43341 | 3575.1 | 75 | 43512 | 3592.2 |
|  |  | 100 | 41690 | 3410 | 25 | 41807 | 3421.7 | 141.2 | 75 | 43319 | 3572.9 | 100 | 43490 | 3590 |
|  |  | 75 | 41668 | 3407.8 | 100 | 41839 | 3424.9 | 140.9 | 25 | 43373 | 3578.3 | 100 | 43490 | 3590 |
|  |  | 100 | 41690 | 3410 | 75 | 41861 | 3427.1 | 140.4 | 100 | 43440 | 3585 | 25 | 43557 | 3596.7 |
|  |  | 75 | 41668 | 3407.8 | 100 | 41839 | 3424.9 | 140.1 | 100 | 43440 | 3585 | 25 | 43557 | 3596.7 |
|  |  | 100 | 41690 | 3410 | 75 | 41861 | 3427.1 | 141.2 | 25 | 43373 | 3578.3 | 100 | 43490 | 3590 |
|  | 25+100+ 100+100 | 25 | 41623 | 3403.3 | 100 | 41740 | 3415 | 135.2 | 100 | 43292 | 3570.2 | 100 | 43490 | 3590 |
|  |  | 100 | 41690 | 3410 | 25 | 41807 | 3421.7 | 136 | 100 | 43292 | 3570.2 | 100 | 43490 | 3590 |
|  |  | 100 | 41690 | 3410 | 100 | 41888 | 3429.8 | 136 | 25 | 43373 | 3578.3 | 100 | 43490 | 3590 |
|  |  | 100 | 41690 | 3410 | 100 | 41888 | 3429.8 | 135.2 | 100 | 43440 | 3585 | 25 | 43557 | 3596.7 |
|  | 50+50+ 100+100 | 50 | 41645 | 3405.5 | 100 | 41789 | 3419.9 | 140.7 | 50 | 43346 | 3575.6 | 100 | 43490 | 3590 |
|  |  | 100 | 41690 | 3410 | 50 | 41834 | 3424.4 | 140.7 | 100 | 43391 | 3580.1 | 50 | 43535 | 3594.5 |
|  |  | 50 | 41645 | 3405.5 | 100 | 41789 | 3419.9 | 140.2 | 100 | 43391 | 3580.1 | 50 | 43535 | 3594.5 |
|  |  | 100 | 41690 | 3410 | 50 | 41834 | 3424.4 | 141.2 | 50 | 43346 | 3575.6 | 100 | 43490 | 3590 |
|  | 50+75+ 100+100 | 50 | 41645 | 3405.5 | 100 | 41789 | 3419.9 | 135.5 | 75 | 43319 | 3572.9 | 100 | 43490 | 3590 |
|  |  | 100 | 41690 | 3410 | 50 | 41834 | 3424.4 | 135.7 | 100 | 43341 | 3575.1 | 75 | 43512 | 3592.2 |
|  |  | 50 | 41645 | 3405.5 | 100 | 41789 | 3419.9 | 135.2 | 100 | 43341 | 3575.1 | 75 | 43512 | 3592.2 |
|  |  | 100 | 41690 | 3410 | 50 | 41834 | 3424.4 | 136 | 75 | 43319 | 3572.9 | 100 | 43490 | 3590 |
|  |  | 75 | 41668 | 3407.8 | 100 | 41839 | 3424.9 | 135.7 | 50 | 43346 | 3575.6 | 100 | 43490 | 3590 |
|  |  | 100 | 41690 | 3410 | 75 | 41861 | 3427.1 | 135.5 | 100 | 43391 | 3580.1 | 50 | 43535 | 3594.5 |
|  |  | 75 | 41668 | 3407.8 | 100 | 41839 | 3424.9 | 135.2 | 100 | 43391 | 3580.1 | 50 | 43535 | 3594.5 |
|  |  | 100 | 41690 | 3410 | 75 | 41861 | 3427.1 | 136 | 50 | 43346 | 3575.6 | 100 | 43490 | 3590 |
|  | 50+100+ 100+100 | 50 | 41645 | 3405.5 | 100 | 41789 | 3419.9 | 130.3 | 100 | 43292 | 3570.2 | 100 | 43490 | 3590 |
|  |  | 100 | 41690 | 3410 | 50 | 41834 | 3424.4 | 130.8 | 100 | 43292 | 3570.2 | 100 | 43490 | 3590 |
|  |  | 100 | 41690 | 3410 | 100 | 41888 | 3429.8 | 130.8 | 50 | 43346 | 3575.6 | 100 | 43490 | 3590 |
|  |  | 100 | 41690 | 3410 | 100 | 41888 | 3429.8 | 130.3 | 100 | 43391 | 3580.1 | 50 | 43535 | 3594.5 |
|  | 75+75+ 100+100 | 75 | 41668 | 3407.8 | 100 | 41839 | 3424.9 | 130.5 | 75 | 43319 | 3572.9 | 100 | 43490 | 3590 |
|  |  | 100 | 41690 | 3410 | 75 | 41861 | 3427.1 | 130.5 | 100 | 43341 | 3575.1 | 75 | 43512 | 3592.2 |
|  |  | 75 | 41668 | 3407.8 | 100 | 41839 | 3424.9 | 130.2 | 100 | 43341 | 3575.1 | 75 | 43512 | 3592.2 |
|  |  | 100 | 41690 | 3410 | 75 | 41861 | 3427.1 | 130.8 | 75 | 43319 | 3572.9 | 100 | 43490 | 3590 |
|  | 75+100+ 100+100 | 75 | 41668 | 3407.8 | 100 | 41839 | 3424.9 | 125.3 | 100 | 43292 | 3570.2 | 100 | 43490 | 3590 |
|  |  | 100 | 41690 | 3410 | 75 | 41861 | 3427.1 | 125.6 | 100 | 43292 | 3570.2 | 100 | 43490 | 3590 |
|  |  | 100 | 41690 | 3410 | 100 | 41888 | 3429.8 | 125.6 | 75 | 43319 | 3572.9 | 100 | 43490 | 3590 |
|  |  | 100 | 41690 | 3410 | 100 | 41888 | 3429.8 | 125.3 | 100 | 43341 | 3575.1 | 75 | 43512 | 3592.2 |
|  | 100+100+ 100+100 | 100 | 41690 | 3410 | 100 | 41888 | 3429.8 | 120.4 | 100 | 43292 | 3570.2 | 100 | 43490 | 3590 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.10A-7: Test frequencies for CA_42A-42D

| Test frequency ID | CC-Combo / |  | CC1 |  | Wgap |  | CC2 |  |  | CC3 |  |  | CC4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB_agg |  | Note1 |  | [MHz] |  | Note1 |  |  | Note1 |  |  | Note1 |  |
|  | [RB] | BW | NUL/DL | fUL/DL |  | BW | NUL/DL | fUL/DL | BW | NUL/DL | fUL/DL | BW | NUL/DL | fUL/DL |
|  |  | [RB] |  | [MHz] |  | [RB] |  | [MHz] | [RB] |  | [MHz] | [RB] |  | [MHz] |
| Max Wgap | 25+25+ 100+100 | 25 | 41615 | 3402.5 | 151 | 25 | 43175 | 3558.5 | 100 | 43292 | 3570.2 | 100 | 43490 | 3590 |
|  |  | 25 | 41615 | 3402.5 | 150.2 | 100 | 43242 | 3565.2 | 100 | 43440 | 3585 | 25 | 43557 | 3596.7 |
|  | 25+50+ 100+100 | 25 | 41615 | 3402.5 | 145.8 | 50 | 43148 | 3555.8 | 100 | 43292 | 3570.2 | 100 | 43490 | 3590 |
|  |  | 25 | 41615 | 3402.5 | 145.3 | 100 | 43193 | 3560.3 | 100 | 43391 | 3580.1 | 50 | 43535 | 3594.5 |
|  |  | 50 | 41640 | 3405 | 146 | 25 | 43175 | 3558.5 | 100 | 43292 | 3570.2 | 100 | 43490 | 3590 |
|  |  | 50 | 41640 | 3405 | 145.2 | 100 | 43242 | 3565.2 | 100 | 43440 | 3585 | 25 | 43557 | 3596.7 |
|  | 25+75+ 100+100 | 25 | 41615 | 3402.5 | 140.6 | 75 | 43121 | 3553.1 | 100 | 43292 | 3570.2 | 100 | 43490 | 3590 |
|  |  | 25 | 41615 | 3402.5 | 140.3 | 100 | 43143 | 3555.3 | 100 | 43341 | 3575.1 | 75 | 43512 | 3592.2 |
|  |  | 75 | 41665 | 3407.5 | 141 | 25 | 43175 | 3558.5 | 100 | 43292 | 3570.2 | 100 | 43490 | 3590 |
|  |  | 75 | 41665 | 3407.5 | 140.2 | 100 | 43242 | 3565.2 | 100 | 43440 | 3585 | 25 | 43557 | 3596.7 |
|  | 25+100+ 100+100 | 25 | 41615 | 3402.5 | 135.4 | 100 | 43094 | 3550.4 | 100 | 43292 | 3570.2 | 100 | 43490 | 3590 |
|  |  | 100 | 41690 | 3410 | 136 | 25 | 43175 | 3558.5 | 100 | 43292 | 3570.2 | 100 | 43490 | 3590 |
|  |  | 100 | 41690 | 3410 | 135.2 | 100 | 43242 | 3565.2 | 100 | 43440 | 3585 | 25 | 43557 | 3596.7 |
|  | 50+50+ 100+100 | 50 | 41640 | 3405 | 140.8 | 50 | 43148 | 3555.8 | 100 | 43292 | 3570.2 | 100 | 43490 | 3590 |
|  |  | 50 | 41640 | 3405 | 140.3 | 100 | 43193 | 3560.3 | 100 | 43391 | 3580.1 | 50 | 43535 | 3594.5 |
|  | 50+75+ 100+100 | 50 | 41640 | 3405 | 135.6 | 75 | 43121 | 3553.1 | 100 | 43292 | 3570.2 | 100 | 43490 | 3590 |
|  |  | 50 | 41640 | 3405 | 135.3 | 100 | 43143 | 3555.3 | 100 | 43341 | 3575.1 | 75 | 43512 | 3592.2 |
|  |  | 75 | 41665 | 3407.5 | 135.8 | 50 | 43148 | 3555.8 | 100 | 43292 | 3570.2 | 100 | 43490 | 3590 |
|  |  | 75 | 41665 | 3407.5 | 135.3 | 100 | 43193 | 3560.3 | 100 | 43391 | 3580.1 | 50 | 43535 | 3594.5 |
|  | 50+100+ 100+100 | 50 | 41640 | 3405 | 130.4 | 100 | 43094 | 3550.4 | 100 | 43292 | 3570.2 | 100 | 43490 | 3590 |
|  |  | 100 | 41690 | 3410 | 130.8 | 50 | 43148 | 3555.8 | 100 | 43292 | 3570.2 | 100 | 43490 | 3590 |
|  |  | 100 | 41690 | 3410 | 130.3 | 100 | 43193 | 3560.3 | 100 | 43391 | 3580.1 | 50 | 43535 | 3594.5 |
|  | 75+75+ 100+100 | 75 | 41665 | 3407.5 | 130.6 | 75 | 43121 | 3553.1 | 100 | 43292 | 3570.2 | 100 | 43490 | 3590 |
|  |  | 75 | 41665 | 3407.5 | 130.3 | 100 | 43143 | 3555.3 | 100 | 43341 | 3575.1 | 75 | 43512 | 3592.2 |
|  | 75+100+ 100+100 | 75 | 41665 | 3407.5 | 125.4 | 100 | 43094 | 3550.4 | 100 | 43292 | 3570.2 | 100 | 43490 | 3590 |
|  |  | 100 | 41690 | 3410 | 125.6 | 75 | 43121 | 3553.1 | 100 | 43292 | 3570.2 | 100 | 43490 | 3590 |
|  |  | 100 | 41690 | 3410 | 125.3 | 100 | 43143 | 3555.3 | 100 | 43341 | 3575.1 | 75 | 43512 | 3592.2 |
|  | 100+100+ 100+100 | 100 | 41690 | 3410 | 120.4 | 100 | 43094 | 3550.4 | 100 | 43292 | 3570.2 | 100 | 43490 | 3590 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.10A-8: Test frequencies for CA_42D-42A

| Test frequency ID | CC-Combo / |  | CC1 |  |  | CC2 |  |  | CC3 |  | Wgap |  | CC4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB_agg |  | Note1 |  |  | Note1 |  |  | Note1 |  | [MHz] |  | Note1 |  |
|  | [RB] | BW | NUL/DL | fUL/DL | BW | NUL/DL | fUL/DL | BW | NUL/DL | fUL/DL |  | BW | NUL/DL | fUL/DL |
|  |  | [RB] |  | [MHz] | [RB] |  | [MHz] | [RB] |  | [MHz] |  | [RB] |  | [MHz] |
| Max Wgap | 25+25+ 100+100 | 25 | 41623 | 3403.3 | 100 | 41740 | 3415 | 100 | 41938 | 3434.8 | 150.2 | 25 | 43565 | 3597.5 |
|  |  | 100 | 41690 | 3410 | 100 | 41888 | 3429.8 | 25 | 42005 | 3441.5 | 151 | 25 | 43565 | 3597.5 |
|  | 25+50+ 100+100 | 50 | 41645 | 3405.5 | 100 | 41789 | 3419.9 | 100 | 41987 | 3439.7 | 145.3 | 25 | 43565 | 3597.5 |
|  |  | 100 | 41690 | 3410 | 100 | 41888 | 3429.8 | 50 | 42032 | 3444.2 | 145.8 | 25 | 43565 | 3597.5 |
|  |  | 25 | 41623 | 3403.3 | 100 | 41740 | 3415 | 100 | 41938 | 3434.8 | 145.2 | 50 | 43540 | 3595 |
|  |  | 100 | 41690 | 3410 | 100 | 41888 | 3429.8 | 25 | 42005 | 3441.5 | 146 | 50 | 43540 | 3595 |
|  | 25+75+ 100+100 | 75 | 41668 | 3407.8 | 100 | 41839 | 3424.9 | 100 | 42037 | 3444.7 | 140.3 | 25 | 43565 | 3597.5 |
|  |  | 100 | 41690 | 3410 | 100 | 41888 | 3429.8 | 75 | 42059 | 3446.9 | 140.6 | 25 | 43565 | 3597.5 |
|  |  | 25 | 41623 | 3403.3 | 100 | 41740 | 3415 | 100 | 41938 | 3434.8 | 140.2 | 75 | 43515 | 3592.5 |
|  |  | 100 | 41690 | 3410 | 100 | 41888 | 3429.8 | 25 | 42005 | 3441.5 | 141 | 75 | 43515 | 3592.5 |
|  | 25+100+ 100+100 | 100 | 41690 | 3410 | 100 | 41888 | 3429.8 | 100 | 42086 | 3449.6 | 135.4 | 25 | 43565 | 3597.5 |
|  |  | 25 | 41623 | 3403.3 | 100 | 41740 | 3415 | 100 | 41938 | 3434.8 | 135.2 | 100 | 43490 | 3590 |
|  |  | 100 | 41690 | 3410 | 100 | 41888 | 3429.8 | 25 | 42005 | 3441.5 | 136 | 100 | 43490 | 3590 |
|  | 50+50+ 100+100 | 50 | 41645 | 3405.5 | 100 | 41789 | 3419.9 | 100 | 41987 | 3439.7 | 140.3 | 50 | 43540 | 3595 |
|  |  | 100 | 41690 | 3410 | 100 | 41888 | 3429.8 | 50 | 42032 | 3444.2 | 140.8 | 50 | 43540 | 3595 |
|  | 50+75+ 100+100 | 75 | 41668 | 3407.8 | 100 | 41839 | 3424.9 | 100 | 42037 | 3444.7 | 135.3 | 50 | 43540 | 3595 |
|  |  | 100 | 41690 | 3410 | 100 | 41888 | 3429.8 | 75 | 42059 | 3446.9 | 135.6 | 50 | 43540 | 3595 |
|  |  | 50 | 41645 | 3405.5 | 100 | 41789 | 3419.9 | 100 | 41987 | 3439.7 | 135.3 | 75 | 43515 | 3592.5 |
|  |  | 100 | 41690 | 3410 | 100 | 41888 | 3429.8 | 50 | 42032 | 3444.2 | 135.8 | 75 | 43515 | 3592.5 |
|  | 50+100+ 100+100 | 100 | 41690 | 3410 | 100 | 41888 | 3429.8 | 100 | 42086 | 3449.6 | 130.4 | 50 | 43540 | 3595 |
|  |  | 50 | 41645 | 3405.5 | 100 | 41789 | 3419.9 | 100 | 41987 | 3439.7 | 130.3 | 100 | 43490 | 3590 |
|  |  | 100 | 41690 | 3410 | 100 | 41888 | 3429.8 | 50 | 42032 | 3444.2 | 130.8 | 100 | 43490 | 3590 |
|  | 75+75+ 100+100 | 75 | 41668 | 3407.8 | 100 | 41839 | 3424.9 | 100 | 42037 | 3444.7 | 130.3 | 75 | 43515 | 3592.5 |
|  |  | 100 | 41690 | 3410 | 100 | 41888 | 3429.8 | 75 | 42059 | 3446.9 | 130.6 | 75 | 43515 | 3592.5 |
|  | 75+100+ 100+100 | 100 | 41690 | 3410 | 100 | 41888 | 3429.8 | 100 | 42086 | 3449.6 | 125.4 | 75 | 43515 | 3592.5 |
|  |  | 75 | 41668 | 3407.8 | 100 | 41839 | 3424.9 | 100 | 42037 | 3444.7 | 125.3 | 100 | 43490 | 3590 |
|  |  | 100 | 41690 | 3410 | 100 | 41888 | 3429.8 | 75 | 42059 | 3446.9 | 125.6 | 100 | 43490 | 3590 |
|  | 100+100+ 100+100 | 100 | 41690 | 3410 | 100 | 41888 | 3429.8 | 100 | 42086 | 3449.6 | 120.4 | 100 | 43490 | 3590 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.10A-9: Test frequencies for CA_42E

| Range | CC-Combo / NRB_agg [RB] |  | CC1 Note1 |  |  | CC2 Note1 |  | CC3 Note1 |  |  | CC4 Note1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL/DL | fUL/DL [MHz] | BW [RB] | NUL/DL | fUL/DL [MHz] | BW [RB] | NUL/DL | fUL/DL [MHz] | BW [RB] | NUL/DL | fUL/DL [MHz] |
| Low | 25+100+100+100 | 25 | 41623 | 3403.3 | 100 | 41740 | 3415 | 100 | 41938 | 3434.8 | 100 | 42136 | 3454.6 |
|  |  | 100 | 41690 | 3410 | 100 | 41888 | 3429.8 | 100 | 42086 | 3449.6 | 25 | 42203 | 3461.3 |
|  | 50+100+100+100 | 50 | 41645 | 3405.5 | 100 | 41789 | 3419.9 | 100 | 41987 | 3439.7 | 100 | 42185 | 3459.5 |
|  |  | 100 | 41690 | 3410 | 100 | 41888 | 3429.8 | 100 | 42086 | 3449.6 | 50 | 42230 | 3464 |
|  | 75+100+100+100 | 75 | 41668 | 3407.8 | 100 | 41839 | 3424.9 | 100 | 42037 | 3444.7 | 100 | 42235 | 3464.5 |
|  |  | 100 | 41690 | 3410 | 100 | 41888 | 3429.8 | 100 | 42086 | 3449.6 | 75 | 42257 | 3466.7 |
|  | 100+100+100+100 | 100 | 41690 | 3410 | 100 | 41888 | 3429.8 | 100 | 42086 | 3449.6 | 100 | 42284 | 3469.4 |
| Mid | 25+100+100+100 | 25 | 42300 | 3471 | 100 | 42417 | 3482.7 | 100 | 42615 | 3502.5 | 100 | 42813 | 3522.3 |
|  |  | 100 | 42367 | 3477.7 | 100 | 42565 | 3497.5 | 100 | 42763 | 3517.3 | 25 | 42880 | 3529 |
|  | 50+100+100+100 | 50 | 42298 | 3470.8 | 100 | 42442 | 3485.2 | 100 | 42640 | 3505 | 100 | 42838 | 3524.8 |
|  |  | 100 | 42343 | 3475.3 | 100 | 42541 | 3495.1 | 100 | 42739 | 3514.9 | 50 | 42883 | 3529.3 |
|  | 75+100+100+100 | 75 | 42295 | 3470.5 | 100 | 42466 | 3487.6 | 100 | 42664 | 3507.4 | 100 | 42862 | 3527.2 |
|  |  | 100 | 42318 | 3472.8 | 100 | 42516 | 3492.6 | 100 | 42714 | 3512.4 | 75 | 42885 | 3529.5 |
|  | 100+100+100+100 | 100 | 42293 | 3470.3 | 100 | 42491 | 3490.1 | 100 | 42689 | 3509.9 | 100 | 42887 | 3529.7 |
| High | 25+100+100+100 | 25 | 42977 | 3538.7 | 100 | 43094 | 3550.4 | 100 | 43292 | 3570.2 | 100 | 43490 | 3590 |
|  |  | 100 | 43044 | 3545.4 | 100 | 43242 | 3565.2 | 100 | 43440 | 3585 | 25 | 43557 | 3596.7 |
|  | 50+100+100+100 | 50 | 42950 | 3536 | 100 | 43094 | 3550.4 | 100 | 43292 | 3570.2 | 100 | 43490 | 3590 |
|  |  | 100 | 42995 | 3540.5 | 100 | 43193 | 3560.3 | 100 | 43391 | 3580.1 | 50 | 43535 | 3594.5 |
|  | 75+100+100+100 | 75 | 42923 | 3533.3 | 100 | 43094 | 3550.4 | 100 | 43292 | 3570.2 | 100 | 43490 | 3590 |
|  |  | 100 | 42945 | 3535.5 | 100 | 43143 | 3555.3 | 100 | 43341 | 3575.1 | 75 | 43512 | 3592.2 |
|  | 100+100+100+100 | 100 | 42896 | 3530.6 | 100 | 43094 | 3550.4 | 100 | 43292 | 3570.2 | 100 | 43490 | 3590 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### 4.3.1.2.11	TDD reference test frequencies for Operating Band 43

Table 4.3.1.2.11-1: Test frequencies for E-UTRA channel bandwidth for operating band 43

| Test Frequency ID | Bandwidth [MHz] | EARFCN | Frequency (UL and DL) [MHz] |
| --- | --- | --- | --- |
| Low Range | 5 | 43615 | 3602.5 |
|  | 10 | 43640 | 3605 |
|  | 15 | 43665 | 3607.5 |
|  | 20 | 43690 | 3610 |
| Mid Range | 5/10/15/20 | 44590 | 3700 |
| High Range | 5 | 45565 | 3797.5 |
|  | 10 | 45540 | 3795 |
|  | 15 | 45515 | 3792.5 |
|  | 20 | 45490 | 3790 |

##### 4.3.1.2.12	TDD reference test frequencies for Operating Band 44

Table 4.3.1.2.12-1: Test frequencies for E-UTRA channel bandwidth for operating band 44

| Test Frequency ID | Bandwidth [MHz] | EARFCN | Frequency (UL and DL) [MHz] |
| --- | --- | --- | --- |
| Low Range | 3 | 45605 | 704.5 |
|  | 5 | 45615 | 705.5 |
|  | 10 | 45640 | 708 |
|  | 15 | 45665 | 710.5 |
|  | 20 | 45690 | 713 |
| Mid Range | 3/5/10/15/20 | 46090 | 753 |
| High Range | 3 | 46575 | 801.5 |
|  | 5 | 46565 | 800.5 |
|  | 10 | 46540 | 798 |
|  | 15 | 46515 | 795.5 |
|  | 20 | 46490 | 793 |

##### 4.3.1.2.13	TDD reference test frequencies for Operating Band 45

Table 4.3.1.2.13-1: Test frequencies for E-UTRA channel bandwidth for operating band 45

| Test Frequency ID | Bandwidth [MHz] | EARFCN | Frequency (UL and DL) [MHz] |
| --- | --- | --- | --- |
| Low Range | 5 | 46615 | 1449.5 |
|  | 10 | 46640 | 1452 |
|  | 15 | 46665 | 1454.5 |
|  | 20 | 46690 | 1457 |
| Mid Range | 5/10/15/20 | 46690 | 1457 |
| High Range | 5 | 46765 | 1464.5 |
|  | 10 | 46740 | 1462 |
|  | 15 | 46715 | 1459.5 |
|  | 20 | 46690 | 1457 |

##### 4.3.1.2.14	TDD reference test frequencies for Operating Band 46

Table 4.3.1.2.14-1: Test frequencies for E-UTRA channel bandwidth for operating band 46

| Test Frequency ID | Bandwidth [MHz] | EARFCN | Frequency (UL and DL) [MHz] |
| --- | --- | --- | --- |
| Low Range | 20 | 46890 | 5160 |
| Mid Range | 20 | 50690 | 5540 |
| High Range | 20 | 54340 | 5905 |
| NOTE 1: In Rel-13, restricted to E-UTRA operation when carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. |  |  |  |

##### 4.3.1.2.14A	TDD reference test frequencies for CA in operating Band 46

Table 4.3.1.2.14A-1: Test frequencies for CA_46C

| Range | CC-Combo / NRB_agg [RB] |  | CC1 Note1 |  |  | CC2 Note1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL/DL | fUL/DL [MHz] | BW [RB] | NUL/DL | fUL/DL [MHz] |
| Low | 100+100 | 100 | 46890 | 5160.0 | 100 | 47088 | 5179.8 |
| Mid | 100+100 | 100 | 50692 | 5540.2 | 100 | 50890 | 5560 |
| High | 100+100 | 100 | 54142 | 5885.2 | 100 | 54340 | 5905 |
| NOTE 1: Carriers in increasing frequency order. NOTE 2: In Rel-13, restricted to E-UTRA operation when carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. |  |  |  |  |  |  |  |

Table 4.3.1.2.14A-2: Test frequencies for CA_46D

| Range | CC-Combo / NRB_agg [RB] |  | CC1 Note1 |  |  | CC2 Note1 |  | CC3 Note1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL/DL | fUL/DL [MHz] | BW [RB] | NUL/DL | fUL/DL [MHz] | BW [RB] | NUL/DL | fUL/DL [MHz] |
| Low | 100+100+100 | 100 | 46892 | 5160.2 | 100 | 47090 | 5180 | 100 | 47288 | 5199.8 |
| Mid | 100+100+100 | 100 | 50492 | 5520.2 | 100 | 50690 | 5540 | 100 | 50888 | 5559.8 |
| High | 100+100+100 | 100 | 53942 | 5865.2 | 100 | 54140 | 5885 | 100 | 54338 | 5904.8 |
| NOTE 1: Carriers in increasing frequency order. NOTE 2: In Rel-13, restricted to E-UTRA operation when carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.14A-3: Test frequencies for CA_46E

| Range | CC-Combo / NRB_agg [RB] |  | CC1 Note1 |  |  | CC2 Note1 |  | CC3 Note1 |  |  |  | CC4 Note1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL/DL | fUL/DL [MHz] | BW [RB] | NUL/DL | fUL/DL [MHz] | BW [RB] | NUL/DL | fUL/DL [MHz] | BW [RB] | NUL/DL | fUL/DL [MHz] |
| Low | 100+100+100+100 | 100 | 46890 | 5160 | 100 | 47088 | 5179.8 | 100 | 47289 | 5199.9 | 100 | 47490 | 5220 |
| Mid | 100+100+100+100 | 100 | 50490 | 5520 | 100 | 50688 | 5539.8 | 100 | 50889 | 5559.9 | 100 | 51090 | 5580 |
| High | 100+100+100+100 | 100 | 53740 | 5845 | 100 | 53941 | 5865.1 | 100 | 54142 | 5885.2 | 100 | 54340 | 5905 |
| NOTE 1: Carriers in increasing frequency order. NOTE 2: In Rel-13, restricted to E-UTRA operation when carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.14A-4: Test frequencies for CA_46A-46A

| Test Frequency ID | CC-Combo / NRB_agg [RB] |  | CC1 Note1 |  | Wgap [MHz] |  | CC2 Note1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL/DL | fUL/DL [MHz] |  | BW [RB] | NUL/DL | fUL/DL [MHz] |
| Max WGap | 100+100 | 100 | 46890 | 5160 | 745 | 100 | 54340 | 5905 |
| NOTE 1: Carriers in increasing frequency order. NOTE 2: In Rel-13, restricted to E-UTRA operation when carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. |  |  |  |  |  |  |  |  |

##### 4.3.1.2.15	TDD reference test frequencies for Operating Band 47

Table 4.3.1.2.15-1: Test frequencies for E-UTRA channel bandwidth for operating band 47

| Test Frequency ID | Bandwidth [MHz] | EARFCN | Frequency (UL and DL) [MHz] |
| --- | --- | --- | --- |
| Low Range | 10 | 54590 | 5860 |
|  | 20 | 54640 | 5865 |
| Mid Range | 10/20 | 54890 | 5890 |
| High Range | 10 | 55190 | 5920 |
|  | 20 | 55140 | 5915 |

Table 4.3.1.2.15-2: Test frequencies for V2X_47B

| Range | CC-Combo / NRB_agg [RB] |  | CC1 Note1 |  |  | CC2 Note1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL/DL | fUL/DL [MHz] | BW [RB] | NUL/DL | fUL/DL [MHz] |
| Low | 50+50 | 50 | 54590 | 5860 | 50 | 54689 | 5869.9 |
| Mid | 50+50 | 50 | 54841 | 5885.1 | 50 | 54940 | 5895 |
| High | 50+50 | 50 | 55091 | 5910.1 | 50 | 55190 | 5920 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |

##### 4.3.1.2.16	TDD reference test frequencies for Operating Band 48

Table 4.3.1.2.16-1: Test frequencies for E-UTRA channel bandwidth for operating band 48

| Test Frequency ID | Bandwidth [MHz] | EARFCN | Frequency (UL and DL) [MHz] |
| --- | --- | --- | --- |
| Low Range | 5 | 55265 | 3552.5 |
|  | 10 | 55290 | 3555.0 |
|  | 15 | 55315 | 3557.5 |
|  | 20 | 55340 | 3560.0 |
| Mid Range | 5/10/15/20 | 55990 | 3625.0 |
| High Range | 5 | 56715 | 3697.5 |
|  | 10 | 56690 | 3695.0 |
|  | 15 | 56665 | 3692.5 |
|  | 20 | 56640 | 3690.0 |

##### 4.3.1.2.16A	TDD reference test frequencies for CA in operating band 48

Table 4.3.1.2.16A-1: Test frequencies for CA_48C

| Range | CC-Combo / NRB_agg [RB] |  | CC1 Note1 |  |  | CC2 Note1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL/DL | fUL/DL [MHz] | BW [RB] | NUL/DL | fUL/DL [MHz] |
| Low | 25+100 | 25 | 55273 | 3553.3 | 100 | 55390 | 3565 |
|  |  | 100 | 55340 | 3560 | 25 | 55457 | 3571.7 |
|  | 50+100 | 50 | 55295 | 3555.5 | 100 | 55439 | 3569.9 |
|  |  | 100 | 55340 | 3560 | 50 | 55484 | 3574.4 |
|  | 75+100 | 75 | 55318 | 3557.8 | 100 | 55489 | 3574.9 |
|  |  | 100 | 55340 | 3560 | 75 | 55511 | 3577.1 |
|  | 100+100 | 100 | 55340 | 3560 | 100 | 55538 | 3579.8 |
| Mid | 25+100 | 25 | 55898 | 3615.8 | 100 | 56015 | 3627.5 |
|  |  | 100 | 55965 | 3622.5 | 25 | 56082 | 3634.2 |
|  | 50+100 | 50 | 55896 | 3615.6 | 100 | 56040 | 3630 |
|  |  | 100 | 55941 | 3620.1 | 50 | 56085 | 3634.5 |
|  | 75+100 | 75 | 55893 | 3615.3 | 100 | 56064 | 3632.4 |
|  |  | 100 | 55916 | 3617.6 | 75 | 56087 | 3634.7 |
|  | 100+100 | 100 | 55891 | 3615.1 | 100 | 56089 | 3634.9 |
| High | 25+100 | 25 | 56523 | 3678.3 | 100 | 56640 | 3690 |
|  |  | 100 | 56590 | 3685 | 25 | 56707 | 3696.7 |
|  | 50+100 | 50 | 56496 | 3675.6 | 100 | 56640 | 3690 |
|  |  | 100 | 56541 | 3680.1 | 50 | 56685 | 3694.5 |
|  | 75+100 | 75 | 56469 | 3672.9 | 100 | 56640 | 3690 |
|  |  | 100 | 56491 | 3675.1 | 75 | 56662 | 3692.2 |
|  | 100+100 | 100 | 56442 | 3670.2 | 100 | 56640 | 3690 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |

Table 4.3.1.2.16A-2: Test frequencies for CA_48A-48A

| Test Frequency ID | CC-Combo / NRB_agg [RB] |  | CC1 Note1 |  | Wgap [MHz] |  | CC2 Note1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL/DL | fUL/DL [MHz] |  | BW [RB] | NUL/DL | fUL/DL [MHz] |
| Max WGap | 25+25 | 25 | 55265 | 3552.5 | 140 | 25 | 56715 | 3697.5 |
|  | 25+50 | 25 | 55265 | 3552.5 | 135 | 50 | 56690 | 3695 |
|  |  | 50 | 55290 | 3555 | 135 | 25 | 56715 | 3697.5 |
|  | 25+75 | 25 | 55265 | 3552.5 | 130 | 75 | 56665 | 3692.5 |
|  |  | 75 | 55315 | 3557.5 | 130 | 25 | 56715 | 3697.5 |
|  | 50+50 | 50 | 55290 | 3555 | 130 | 50 | 56690 | 3695 |
|  | 25+100 | 25 | 55265 | 3552.5 | 125 | 100 | 56640 | 3690 |
|  |  | 100 | 55340 | 3560 | 125 | 25 | 56715 | 3697.5 |
|  | 50+75 | 50 | 55290 | 3555 | 125 | 75 | 56665 | 3692.5 |
|  |  | 75 | 55315 | 3557.5 | 125 | 50 | 56690 | 3695 |
|  | 50+100 | 50 | 55290 | 3555 | 120 | 100 | 56640 | 3690 |
|  |  | 100 | 55340 | 3560 | 120 | 50 | 56690 | 3695 |
|  | 75+100 | 75 | 55315 | 3557.5 | 115 | 100 | 56640 | 3690 |
|  |  | 100 | 55340 | 3560 | 115 | 75 | 56665 | 3692.5 |
|  | 100+100 | 100 | 55340 | 3560 | 110 | 100 | 56640 | 3690 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |

Table 4.3.1.2.16A-3: Test frequencies for CA_48D

| Range | CC-Combo / NRB_agg [RB] |  | CC1 Note1 |  |  | CC2 Note1 |  | CC3 Note1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW [RB] | NUL/DL | fUL/DL [MHz] | BW [RB] | NUL/DL | fUL/DL [MHz] | BW [RB] | NUL/DL | fUL/DL [MHz] |
| Low | 25+100+100 | 25 | 55273 | 3553.3 | 100 | 55390 | 3565 | 100 | 55588 | 3584.8 |
|  |  | 100 | 55340 | 3560 | 100 | 55538 | 3579.8 | 25 | 55655 | 3591.5 |
|  | 50+100+100 | 50 | 55295 | 3555.5 | 100 | 55439 | 3569.9 | 100 | 55637 | 3589.7 |
|  |  | 100 | 55340 | 3560 | 100 | 55538 | 3579.8 | 50 | 55682 | 3594.2 |
|  | 75+100+100 | 75 | 55318 | 3557.8 | 100 | 55489 | 3574.9 | 100 | 55687 | 3594.7 |
|  |  | 100 | 55340 | 3560 | 100 | 55538 | 3579.8 | 75 | 55709 | 3596.9 |
|  | 100+100+100 | 100 | 55340 | 3560 | 100 | 55538 | 3579.8 | 100 | 55736 | 3599.6 |
| Mid | 25+100+100 | 25 | 55799 | 3605.9 | 100 | 55916 | 3617.6 | 100 | 56114 | 3637.4 |
|  |  | 100 | 55866 | 3612.6 | 100 | 56064 | 3632.4 | 25 | 56181 | 3644.1 |
|  | 50+100+100 | 50 | 55797 | 3605.7 | 100 | 55941 | 3620.1 | 100 | 56139 | 3639.9 |
|  |  | 100 | 55842 | 3610.2 | 100 | 56040 | 3630 | 50 | 56184 | 3644.4 |
|  | 75+100+100 | 75 | 55794 | 3605.4 | 100 | 55965 | 3622.5 | 100 | 56163 | 3642.3 |
|  |  | 100 | 55817 | 3607.7 | 100 | 56015 | 3627.5 | 75 | 56186 | 3644.6 |
|  | 100+100+100 | 100 | 55792 | 3605.2 | 100 | 55990 | 3625 | 100 | 56188 | 3644.8 |
| High | 25+100+100 | 25 | 56325 | 3658.5 | 100 | 56442 | 3670.2 | 100 | 56640 | 3690 |
|  |  | 100 | 56392 | 3665.2 | 100 | 56590 | 3685 | 25 | 56707 | 3696.7 |
|  | 50+100+100 | 50 | 56298 | 3655.8 | 100 | 56442 | 3670.2 | 100 | 56640 | 3690 |
|  |  | 100 | 56343 | 3660.3 | 100 | 56541 | 3680.1 | 50 | 56685 | 3694.5 |
|  | 75+100+100 | 75 | 56271 | 3653.1 | 100 | 56442 | 3670.2 | 100 | 56640 | 3690 |
|  |  | 100 | 56293 | 3655.3 | 100 | 56491 | 3675.1 | 75 | 56662 | 3692.2 |
|  | 100+100+100 | 100 | 56244 | 3650.4 | 100 | 56442 | 3670.2 | 100 | 56640 | 3690 |
| Note 1: Carriers in increasing frequency order. |  |  |  |  |  |  |  |  |  |  |

##### 4.3.1.2.17	TDD reference test frequencies for Operating Band 53

Table 4.3.1.2.17-1: Test frequencies for E-UTRA channel bandwidth for operating band 53

| Test Frequency ID | Bandwidth [MHz] | EARFCN | Frequency (UL and DL) [MHz] |
| --- | --- | --- | --- |
| Low Range | 1.4 | 60147 | 2484.2 |
|  | 3 | 60155 | 2485 |
|  | 5 | 60165 | 2486 |
|  | 10 | 60190 | 2488.5 |
| Mid Range | 1.4/3/5/10 | 60198 | 2489.3 |
| High Range | 1.4 | 60248 | 2494.3 |
|  | 3 | 60240 | 2493.5 |
|  | 5 | 60230 | 2492.5 |
|  | 10 | 60205 | 2490 |

#### 4.3.1.3	HRPD Test frequencies

##### 4.3.1.3.1	HRPD test frequencies for Band Class 0

Table 4.3.1.3.1-1: Test frequencies for HRPD in Band Class 0

| Test Frequency ID | Bandwidth [MHz] | Channel Number | Frequency of Uplink [MHz] | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- |
| Low Range | 1.23 | 1013 | 824.70 | 869.70 |
| Mid Range | 1.23 | 356 | 835.68 | 880.68 |
| High Range | 1.23 | 779 | 848.37 | 893.37 |

##### 4.3.1.3.2	HRPD test frequencies for Band Class 1

Table 4.3.1.3.2-1: Test frequencies for HRPD in Band Class 1

| Test Frequency ID | Bandwidth [MHz] | Channel Number | Frequency of Uplink [MHz] | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- |
| Low Range | 1.25 | 25 | 1851.25 | 1931.25 |
| Mid Range | 1.25 | 600 | 1880.00 | 1960.00 |
| High Range | 1.25 | 1175 | 1908.75 | 1988.75 |

##### 4.3.1.3.3	HRPD test frequencies for Band Class 3

Table 4.3.1.3.3-1: Test frequencies for HRPD in Band Class 3

| Test Frequency ID | Bandwidth [MHz] | Channel Number | Frequency of Uplink [MHz] | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- |
| Low Range | 1.25 | 1120 | 888.00 | 833.00 |
| Mid Range | 1.25 | 872 | 898.90 | 843.90 |
| High Range | 1.25 | 76 | 915.95 | 860.95 |

##### 4.3.1.3.4	HRPD test frequencies for Band Class 4

Table 4.3.1.3.4-1: Test frequencies for HRPD in Band Class 4

| Test Frequency ID | Bandwidth [MHz] | Channel Number | Frequency of Uplink [MHz] | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- |
| Low Range | 1.25 | 100 | 1755.00 | 1845.00 |
| Mid Range | 1.25 | 300 | 1765.00 | 1855.00 |
| High Range | 1.25 | 525 | 1776.25 | 1866.25 |

##### 4.3.1.3.5	HRPD test frequencies for Band Class 6

Table 4.3.1.3.5-1: Test frequencies for HRPD in Band Class 6

| Test Frequency ID | Bandwidth [MHz] | Channel Number | Frequency of Uplink [MHz] | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- |
| Low Range | 1.25 | 125 | 1926.25 | 2116.25 |
| Mid Range | 1.25 | 200 | 1930.00 | 2120.00 |
| High Range | 1.25 | 350 | 1937.50 | 2127.50 |

##### 4.3.1.3.6	HRPD test frequencies for Band Class 10

Table 4.3.1.3.6-1: Test frequencies for HRPD in Band Class 10

| Test Frequency ID | Bandwidth [MHz] | Channel Number | Frequency of Uplink [MHz] | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- |
| Low Range | 1.25 | 100 | 808.50 | 853.50 |
| Mid Range | 1.25 | 500 | 821.00 | 866.00 |
| High Range | 1.25 | 820 | 898.50 | 937.50 |

##### 4.3.1.3.7	HRPD test frequencies for Band Class 15

Table 4.3.1.3.7-1: Test frequencies for HRPD in Band Class 15

| Test Frequency ID | Bandwidth [MHz] | Channel Number | Frequency of Uplink [MHz] | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- |
| Low Range | 1.25 | 75 | 1713.75 | 2113.75 |
| Mid Range | 1.25 | 450 | 1732.50 | 2132.50 |
| High Range | 1.25 | 800 | 1750.00 | 2150.00 |

#### 4.3.1.4	1xRTT Test frequencies

##### 4.3.1.4.1	1xRTT test frequencies for Band Class 0

Table 4.3.1.4.1-1: Test frequencies for 1xRTT in Band Class 0

| Test Frequency ID | Bandwidth [MHz] | Channel Number | Frequency of Uplink [MHz] | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- |
| Low Range | 1.23 | 1013 | 824.70 | 869.70 |
| Mid Range | 1.23 | 356 | 835.68 | 880.68 |
| High Range | 1.23 | 779 | 848.37 | 893.37 |

##### 4.3.1.4.2	1xRTT test frequencies for Band Class 1

Table 4.3.1.4.2-1: Test frequencies for 1xRTT in Band Class 1

| Test Frequency ID | Bandwidth [MHz] | Channel Number | Frequency of Uplink [MHz] | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- |
| Low Range | 1.25 | 25 | 1851.25 | 1931.25 |
| Mid Range | 1.25 | 600 | 1880.00 | 1960.00 |
| High Range | 1.25 | 1175 | 1908.75 | 1988.75 |

##### 4.3.1.4.3	1xRTT test frequencies for Band Class 3

Table 4.3.1.4.3-1: Test frequencies for 1xRTT in Band Class 3

| Test Frequency ID | Bandwidth [MHz] | Channel Number | Frequency of Uplink [MHz] | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- |
| Low Range | 1.25 | 1120 | 888.00 | 833.00 |
| Mid Range | 1.25 | 872 | 898.90 | 843.90 |
| High Range | 1.25 | 76 | 915.95 | 860.95 |

##### 4.3.1.4.4	1xRTT test frequencies for Band Class 4

Table 4.3.1.4.4-1: Test frequencies for 1xRTT in Band Class 4

| Test Frequency ID | Bandwidth [MHz] | Channel Number | Frequency of Uplink [MHz] | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- |
| Low Range | 1.25 | 100 | 1755.00 | 1845.00 |
| Mid Range | 1.25 | 300 | 1765.00 | 1855.00 |
| High Range | 1.25 | 525 | 1776.25 | 1866.25 |

##### 4.3.1.4.5	1xRTT test frequencies for Band Class 6

Table 4.3.1.4.5-1: Test frequencies for 1xRTT in Band Class 6

| Test Frequency ID | Bandwidth [MHz] | Channel Number | Frequency of Uplink [MHz] | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- |
| Low Range | 1.25 | 225 | 1931.25 | 2121.25 |
| Mid Range | 1.25 | 275 | 1933.75 | 2123.75 |
| High Range | 1.25 | 375 | 1938.75 | 2128.75 |

##### 4.3.1.4.6	1xRTT test frequencies for Band Class 10

Table 4.3.1.4.6-1: Test frequencies for 1xRTT in Band Class 10

| Test Frequency ID | Bandwidth [MHz] | Channel Number | Frequency of Uplink [MHz] | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- |
| Low Range | 1.25 | 100 | 808.50 | 853.50 |
| Mid Range | 1.25 | 500 | 821.00 | 866.00 |
| High Range | 1.25 | 820 | 898.50 | 937.50 |

##### 4.3.1.4.7	1xRTT test frequencies for Band Class 15

Table 4.3.1.4.7-1: Test frequencies for 1xRTT in Band Class 15

| Test Frequency ID | Bandwidth [MHz] | Channel Number | Frequency of Uplink [MHz] | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- |
| Low Range | 1.25 | 75 | 1713.75 | 2113.75 |
| Mid Range | 1.25 | 450 | 1732.50 | 2132.50 |
| High Range | 1.25 | 800 | 1750.00 | 2150.00 |

#### 4.3.1.5	MFBI Test frequencies

NOTE:	MFBI test frequencies are not used for TS 36.521-1, TS 36.521-3 and TS 37.571-1.

##### 4.3.1.5.1	MFBI Test frequencies for operation band 2 overlapping with band 25

- same as per Table 4.3.1.1.2-1

##### 4.3.1.5.2	MFBI Test frequencies for operation band 3 overlapping with band 9

Table 4.3.1.5.2-1: Test frequencies for E-UTRA channel bandwidth for operating band 3 overlapping with band 9

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 1.4 | 19606 | 1750.6 | 1606 | 1845.6 |
|  | 3 | 19614 | 1751.4 | 1614 | 1846.4 |
|  | 5 | 19624 | 1752.4 | 1624 | 1847.4 |
|  | 10 | 19649 | 1754.9 | 1649 | 1849.9 |
|  | 15 [1] | 19674 | 1757.4 | 1674 | 1852.4 |
|  | 20 [1] | 19699 | 1759.9 | 1699 | 1854.9 |
| Mid Range | 1.4/3/5/10 15 [1]/20 [1] | 19774 | 1767.4 | 1774 | 1862.4 |
| High Range | 1.4 | 19942 | 1784.2 | 1942 | 1879.2 |
|  | 3 | 19934 | 1783.4 | 1934 | 1878.4 |
|  | 5 | 19924 | 1782.4 | 1924 | 1877.4 |
|  | 10 | 19899 | 1779.9 | 1899 | 1874.9 |
|  | 15 [1] | 19874 | 1777.4 | 1874 | 1872.4 |
|  | 20 [1] | 19849 | 1774.9 | 1849 | 1869.9 |
| NOTE 1: Bandwidth for which a relaxation of the specified UE receiver sensitivity requirement (TS 36.101 [27] Clause 7.3) is allowed. |  |  |  |  |  |

##### 4.3.1.5.3	MFBI Test frequencies for operation band 4 overlapping with band 10

- same as per Table 4.3.1.1.4-1.

##### 4.3.1.5.4	MFBI Test frequencies for operation band 5 overlapping with band 18

Table 4.3.1.5.4-1: Test frequencies for E-UTRA channel bandwidth for operating band 5 overlapping with band 18

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 1.4 | 20407 | 824.7 | 2407 | 869.7 |
|  | 3 | 20415 | 825.5 | 2415 | 870.5 |
|  | 5 | 20425 | 826.5 | 2425 | 871.5 |
| Mid Range | 1.4/3/5 | 20430 | 827 | 2430 | 872 |
| High Range | 1.4 | 20453 | 829.3 | 2453 | 874.3 |
|  | 3 | 20445 | 828.5 | 2445 | 873.5 |
|  | 5 | 20435 | 827.5 | 2435 | 872.5 |

##### 4.3.1.5.5	MFBI Test frequencies for operation band 5 overlapping with band 19

Table 4.3.1.5.5-1: Test frequencies for E-UTRA channel bandwidth for operating band 5 overlapping with band 19

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 1.4 | 20467 | 830.7 | 2467 | 875.7 |
|  | 3 | 20475 | 831.5 | 2475 | 876.5 |
|  | 5 | 20485 | 832.5 | 2485 | 877.5 |
|  | 10 [1] | 20510 | 835 | 2510 | 880 |
| Mid Range | 1.4/3/5 10 [1] | 20535 | 837.5 | 2535 | 882.5 |
| High Range | 1.4 | 20603 | 844.3 | 2603 | 889.3 |
|  | 3 | 20595 | 843.5 | 2595 | 888.5 |
|  | 5 | 20585 | 842.5 | 2585 | 887.5 |
|  | 10 [1] | 20560 | 840 | 2560 | 885 |
| NOTE 1: Bandwidth for which a relaxation of the specified UE receiver sensitivity requirement (TS 36.101 [27] Clause 7.3) is allowed. |  |  |  |  |  |

##### 4.3.1.5.6	MFBI Test frequencies for operation band 5 overlapping with band 26

- same as per Table 4.3.1.1.5-1

##### 4.3.1.5.7	MFBI Test frequencies for operation band 9 overlapping with band 3

- same as per Table 4.3.1.1.9-1

##### 4.3.1.5.8	MFBI Test frequencies for operation band 10 overlapping with band 4

Table 4.3.1.5.8-1: Test frequencies for E-UTRA channel bandwidth for operating band 10 overlapping with band 4

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 5 | 22175 | 1712.5 | 4175 | 2112.5 |
|  | 10 | 22200 | 1715 | 4200 | 2115 |
|  | 15 | 22225 | 1717.5 | 4225 | 2117.5 |
|  | 20 | 22250 | 1720 | 4250 | 2120 |
| Mid Range | 5/10/15/20 | 22375 | 1732.5 | 4375 | 2132.5 |
| High Range | 5 | 22575 | 1752.5 | 4575 | 2152.5 |
|  | 10 | 22550 | 1750 | 4550 | 2150 |
|  | 15 | 22525 | 1747.5 | 4525 | 2147.5 |
|  | 20 | 22500 | 1745 | 4500 | 2145 |

##### 4.3.1.5.9	MFBI Test frequencies for operation band 12 overlapping with band 17

Table 4.3.1.5.9-1: Test frequencies for E-UTRA channel bandwidth for operating band 12 overlapping with band 17

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 1.4 | 23067 | 704.7 | 5067 | 734.7 |
|  | 3 | 23075 | 705.5 | 5075 | 735.5 |
|  | 5 [1] | 23085 | 706.5 | 5085 | 736.5 |
|  | 10 [1] | 23110 | 709 | 5110 | 739 |
| Mid Range | 1.4/3 5 [1]/10 [1] | 23120 | 710 | 5120 | 740 |
| High Range | 1.4 | 23173 | 715.3 | 5173 | 745.3 |
|  | 3 | 23165 | 714.5 | 5165 | 744.5 |
|  | 5 [1] | 23155 | 713.5 | 5155 | 743.5 |
|  | 10 [1] | 23130 | 711 | 5130 | 741 |
| NOTE 1: Bandwidth for which a relaxation of the specified UE receiver sensitivity requirement (TS 36.101 [27] Clause 7.3) is allowed. |  |  |  |  |  |

##### 4.3.1.5.10	MFBI Test frequencies for operation band 17 overlapping with band 12

- same as per Table 4.3.1.1.17-1

##### 4.3.1.5.11	MFBI Test frequencies for operation band 18 overlapping with band 5

Table 4.3.1.5.11-1: Test frequencies for E-UTRA channel bandwidth for operating band 18 overlapping with band 5

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 1.4 | 23947 | 824.7 | 5947 | 869.7 |
|  | 3 | 23955 | 825.5 | 5955 | 870.5 |
|  | 5 | 23965 | 826.5 | 5965 | 871.5 |
| Mid Range | 1.4/3/5 | 23970 | 827 | 5970 | 872 |
| High Range | 1.4 | 23993 | 829.3 | 5993 | 874.3 |
|  | 3 | 23985 | 828.5 | 5985 | 873.5 |
|  | 5 | 23975 | 827.5 | 5975 | 872.5 |

##### 4.3.1.5.12	MFBI Test frequencies for operation band 18 overlapping with band 26

- same as per Table 4.3.1.1.18-1

##### 4.3.1.5.13	MFBI Test frequencies for operation band 18 overlapping with band 27

Table 4.3.1.5.13-1: Test frequencies for E-UTRA channel bandwidth for operating band 18 overlapping with band 27

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 1.4 | 23857 | 815.7 | 5857 | 860.7 |
|  | 3 | 23865 | 816.5 | 5865 | 861.5 |
|  | 5 | 23875 | 817.5 | 5875 | 862.5 |
| Mid Range | 1.4/3/5 | 23895 | 819.5 | 5895 | 864.5 |
| High Range | 1.4 | 23933 | 823.3 | 5933 | 868.3 |
|  | 3 | 23925 | 822.5 | 5925 | 867.5 |
|  | 5 | 23915 | 821.5 | 5915 | 866.5 |

##### 4.3.1.5.14	MFBI Test frequencies for operation band 19 overlapping with band 5

- same as per Table 4.3.1.1.19-1

##### 4.3.1.5.15	MFBI Test frequencies for operation band 19 overlapping with band 26

- same as per Table 4.3.1.1.19-1

##### 4.3.1.5.16	MFBI Test frequencies for operation band 25 overlapping with band 2

Table 4.3.1.5.16-1: Test frequencies for E-UTRA channel bandwidth for operating band 25 overlapping with band 2

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 1.4 | 26047 | 1850.7 | 8047 | 1930.7 |
|  | 3 | 26055 | 1851.5 | 8055 | 1931.5 |
|  | 5 | 26065 | 1852.5 | 8065 | 1932.5 |
|  | 10 | 26090 | 1855 | 8090 | 1935 |
|  | 15 [1] | 26115 | 1857.5 | 8115 | 1937.5 |
|  | 20 [1] | 26140 | 1860 | 8140 | 1940 |
| Mid Range | 1.4/3/5/10 15 [1]/20 [1] | 26340 | 1880 | 8340 | 1960 |
| High Range | 1.4 | 26633 | 1909.3 | 8633 | 1989.3 |
|  | 3 | 26625 | 1908.5 | 8625 | 1988.5 |
|  | 5 | 26615 | 1907.5 | 8615 | 1987.5 |
|  | 10 | 26590 | 1905 | 8590 | 1985 |
|  | 15 [1] | 26565 | 1902.5 | 8565 | 1982.5 |
|  | 20 [1] | 26540 | 1900 | 8540 | 1980 |
| NOTE 1: Bandwidth for which a relaxation of the specified UE receiver sensitivity requirement (TS 36.101 [27] Clause 7.3) is allowed. |  |  |  |  |  |

##### 4.3.1.5.17	MFBI Test frequencies for operation band 26 overlapping with band 5

Table 4.3.1.5.17-1: Test frequencies for E-UTRA channel bandwidth for operating band 26 overlapping with band 5

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 1.4 | 26797 | 824.7 | 8797 | 869.7 |
|  | 3 | 26805 | 825.5 | 8805 | 870.5 |
|  | 5 | 26815 | 826.5 | 8815 | 871.5 |
|  | 10[1] | 26840 | 829 | 8840 | 874 |
|  | 15 [1] | 26865 | 831.5 | 8865 | 876.5 |
| Mid Range | 1.4/3/5/10[1] 15 [1] | 26915 | 836.5 | 8915 | 881.5 |
| High Range | 1.4 | 27033 | 848.3 | 9033 | 893.3 |
|  | 3 | 27025 | 847.5 | 9025 | 892.5 |
|  | 5 | 27015 | 846.5 | 9015 | 891.5 |
|  | 10[1] | 26990 | 844 | 8990 | 889 |
|  | 15 [1] | 26965 | 841.5 | 8965 | 886.5 |
| NOTE 1: Bandwidth for which a relaxation of the specified UE receiver sensitivity requirement (TS 36.101 [27] Clause 7.3) is allowed. |  |  |  |  |  |

##### 4.3.1.5.18	MFBI Test frequencies for operation band 26 overlapping with band 18

Table 4.3.1.5.18-1: Test frequencies for E-UTRA channel bandwidth for operating band 26 overlapping with band 18

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 1.4 | 26707 | 815.7 | 8707 | 860.7 |
|  | 3 | 26715 | 816.5 | 8715 | 861.5 |
|  | 5 | 26725 | 817.5 | 8725 | 862.5 |
|  | 10[1] | 26750 | 820 | 8750 | 865 |
| Mid Range | 1.4/3/5/10[1] | 26775 | 822.5 | 8775 | 867.5 |
| High Range | 1.4 | 26843 | 829.3 | 8843 | 874.3 |
|  | 3 | 26835 | 828.5 | 8835 | 873.5 |
|  | 5 | 26825 | 827.5 | 8825 | 872.5 |
|  | 10[1] | 26800 | 825 | 8800 | 870 |
| NOTE 1: Bandwidth for which a relaxation of the specified UE receiver sensitivity requirement (TS 36.101 [27] Clause 7.3) is allowed. |  |  |  |  |  |

##### 4.3.1.5.19	MFBI Test frequencies for operation band 26 overlapping with band 19

Table 4.3.1.5.19-1: Test frequencies for E-UTRA channel bandwidth for operating band 26 overlapping with band 19

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 1.4 | 26857 | 830.7 | 8857 | 875.7 |
|  | 3 | 26865 | 831.5 | 8865 | 876.5 |
|  | 5 | 26875 | 832.5 | 8875 | 877.5 |
|  | 10[1] | 26900 | 835 | 8900 | 880 |
| Mid Range | 1.4/3/5/10[1] 15 [1] | 26925 | 837.5 | 8925 | 882.5 |
| High Range | 1.4 | 26993 | 844.3 | 8993 | 889.3 |
|  | 3 | 26985 | 843.5 | 8985 | 888.5 |
|  | 5 | 26975 | 842.5 | 8975 | 887.5 |
|  | 10[1] | 26950 | 840 | 8950 | 885 |
| NOTE 1: Bandwidth for which a relaxation of the specified UE receiver sensitivity requirement (TS 36.101 [27] Clause 7.3) is allowed. |  |  |  |  |  |

##### 4.3.1.5.20	MFBI Test frequencies for operation band 26 overlapping with band 27

Table 4.3.1.5.20-1: Test frequencies for E-UTRA channel bandwidth for operating band 26 overlapping with band 27

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 1.4 | 26697 | 814.7 | 8697 | 859.7 |
|  | 3 | 26705 | 815.5 | 8705 | 860.5 |
|  | 5 | 26715 | 816.5 | 8715 | 861.5 |
| Mid Range | 1.4/3/5 | 26740 | 819 | 8740 | 864 |
| High Range | 1.4 | 26783 | 823.3 | 8783 | 868.3 |
|  | 3 | 26775 | 822.5 | 8775 | 867.5 |
|  | 5 | 26765 | 821.5 | 8765 | 866.5 |
| NOTE 1: Bandwidth for which a relaxation of the specified UE receiver sensitivity requirement (TS 36.101 [27] Clause 7.3) is allowed. |  |  |  |  |  |

##### 4.3.1.5.21	MFBI Test frequencies for operation band 27 overlapping with band 18

Table 4.3.1.5.21-1: Test frequencies for E-UTRA channel bandwidth for operating band 27 overlapping with band 18

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 1.4 | 27127 | 815.7 | 9127 | 860.7 |
|  | 3 | 27135 | 816.5 | 9135 | 861.5 |
|  | 5 | 27145 | 817.5 | 9145 | 862.5 |
| Mid Range | 1.4/3/5 | 27165 | 819.5 | 9165 | 864.5 |
| High Range | 1.4 | 27203 | 823.3 | 9203 | 868.3 |
|  | 3 | 27195 | 822.5 | 9195 | 867.5 |
|  | 5 | 27185 | 821.5 | 9185 | 866.5 |

##### 4.3.1.5.22	MFBI Test frequencies for operation band 27 overlapping with band 26

Table 4.3.1.5.22-1: Test frequencies for E-UTRA channel bandwidth for operating band 27 overlapping with band 26

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 1.4 | 27117 | 814.7 | 9117 | 859.7 |
|  | 3 | 27125 | 815.5 | 9125 | 860.5 |
|  | 5 | 27135 | 816.5 | 9135 | 861.5 |
| Mid Range | 1.4/3/5 | 27160 | 819 | 9160 | 864 |
| High Range | 1.4 | 27203 | 823.3 | 9203 | 868.3 |
|  | 3 | 27195 | 822.5 | 9195 | 867.5 |
|  | 5 | 27185 | 821.5 | 9185 | 866.5 |
| NOTE 1: Bandwidth for which a relaxation of the specified UE receiver sensitivity requirement (TS 36.101 [27] Clause 7.3) is allowed. |  |  |  |  |  |

##### 4.3.1.5.23	MFBI Test frequencies for operation band 33 overlapping with band 39

- same as per Table 4.3.1.2.1-1

##### 4.3.1.5.24	MFBI Test frequencies for operation band 38 overlapping with band 41

- same as per Table 4.3.1.2.6-1

##### 4.3.1.5.25	MFBI Test frequencies for operation band 39 overlapping with band 33

Table 4.3.1.5.25-1: Test frequencies for E-UTRA channel bandwidth for operating band 39 overlapping with band 33

| Test Frequency ID | Bandwidth [MHz] | EARFCN | Frequency (UL and DL) [MHz] |
| --- | --- | --- | --- |
| Low Range | 5 | 38475 | 1902.5 |
|  | 10 | 38500 | 1905 |
|  | 15 | 38525 | 1907.5 |
|  | 20 | 38550 | 1910 |
| Mid Range | 5/10/15/20 | 38550 | 1910 |
| High Range | 5 | 38625 | 1917.5 |
|  | 10 | 38600 | 1915 |
|  | 15 | 38575 | 1912.5 |
|  | 20 | 38550 | 1910 |

##### 4.3.1.5.26	MFBI Test frequencies for operation band 41 overlapping with band 38

Table 4.3.1.5.26-1: Test frequencies for E-UTRA channel bandwidth for operating band 41 overlapping with band 38

| Test Frequency ID | Bandwidth [MHz] | EARFCN | Frequency (UL and DL) [MHz] |
| --- | --- | --- | --- |
| Low Range | 5 | 40415 | 2572.5 |
|  | 10 | 40440 | 2575 |
|  | 15 | 40465 | 2577.5 |
|  | 20 | 40490 | 2580 |
| Mid Range | 5/10/15/20 | 40640 | 2595 |
| High Range | 5 | 40865 | 2617.5 |
|  | 10 | 40840 | 2615 |
|  | 15 | 40815 | 2612.5 |
|  | 20 | 40790 | 2610 |

##### 4.3.1.5.27	MFBI Test frequencies for operation band 66 overlapping with band 4

Table 4.3.1.5.27-1: Test frequencies for E-UTRA channel bandwidth for operating band 66 overlapping with band 4

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 1.4 | 131979 | 1710.7 | 66443 | 2110.7 |
|  | 3 | 131987 | 1711.5 | 66451 | 2111.5 |
|  | 5 | 131997 | 1712.5 | 66461 | 2112.5 |
|  | 10 | 132022 | 1715 | 66486 | 2115 |
|  | 15 | 132047 | 1717.5 | 66511 | 2117.5 |
|  | 20 | 132072 | 1720 | 66536 | 2120 |
| Mid Range | 1.4/3/5/10/15/20 | 132197 | 1732.5 | 66661 | 2132.5 |
| High Range | 1.4 | 132415 | 1754.3 | 66879 | 2154.3 |
|  | 3 | 132407 | 1753.5 | 66871 | 2153.5 |
|  | 5 | 132397 | 1752.5 | 66861 | 2152.5 |
|  | 10 | 132372 | 1750 | 66836 | 2150 |
|  | 15 | 132347 | 1747.5 | 66811 | 2147.5 |
|  | 20 | 132322 | 1745 | 66786 | 2145 |

##### 4.3.1.5.28	MFBI Test frequencies for operation band 66 overlapping with band 10

Table 4.3.1.5.28-1: Test frequencies for E-UTRA channel bandwidth for operating band 66 overlapping with band 10

| Test Frequency ID | Bandwidth [MHz] | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- |
| Low Range | 5 | 131997 | 1712.5 | 66461 | 2112.5 |
|  | 10 | 132022 | 1715 | 66486 | 2115 |
|  | 15 | 132047 | 1717.5 | 66511 | 2117.5 |
|  | 20 | 132072 | 1720 | 66536 | 2120 |
| Mid Range | 5/10/15/20 | 132197 | 1732.5 | 66736 | 2140 |
| High Range | 5 | 132547 | 1767.5 | 67011 | 2167.5 |
|  | 10 | 132522 | 1765 | 66986 | 2165 |
|  | 15 | 132447 | 1757.5 | 66961 | 2162.5 |
|  | 20 | 132472 | 1760 | 66936 | 2160 |

##### 4.3.1.5.29	MFBI Test frequencies for operation band 4 overlapping with band 66

- same as per Table 4.3.1.1.4-1.

##### 4.3.1.5.30	MFBI Test frequencies for operation band 10 overlapping with band 66

- same as per Table 4.3.1.1.10-1.

#### 4.3.1.6	WLAN Test frequencies

##### 4.3.1.6.1	WLAN Test frequencies for 2.4 GHz ISM Band

Table 4.3.1.6.1-1: Test frequencies for WLAN for 2.4 GHz Band

| Test Frequency ID | Bandwidth [MHz] | WLAN Channel Number | Centre Frequency (UL and DL)[MHz] |
| --- | --- | --- | --- |
| Low Range (1) | 20 | 1 | 2412 |
| Mid Range (2) | 20 | 6 | 2437 |
| High Range (3) | 20 | 11 | 2462 |

##### 4.3.1.6.2	WLAN Test frequencies for 5 GHz ISM Band

Table 4.3.1.6.1-1: Test frequencies for WLAN for 5 GHz Band

| Test Frequency ID | Bandwidth [MHz] | WLAN Channel Number | Centre Frequency (UL and DL)[MHz] |
| --- | --- | --- | --- |
| Low Range (1) | 20 | 48 | 5240 |
| Mid Range (2) | 20 | 56 | 5280 |
| High Range (3) | 20 | 64 | 5320 |

#### 4.3.1.7	Bluetooth Test frequencies

##### 4.3.1.7.1	Bluetooth Test frequencies for 2.4 GHz ISM Band

Table 4.3.1.7.1-1: Test frequencies for Bluetooth for 2.4 GHz Band

| Test Frequency ID | Channel Bandwidth [MHz] | Operating Band (UL and DL)[MHz] |
| --- | --- | --- |
| Mid Range (1) | 1 (Note 1) | 2402 - 2480 |
| Note 1: Bluetooth adopts FHSS to use the Operating Band 2402MHz – 2480MHz. |  |  |

### 4.3.2	Radio conditions

#### 4.3.2.1	Normal propagation condition

The downlink connection between the System Simulator and the UE is without Additive White Gaussian Noise, and has no fading or multipath effects.

The uplink connection between the UE and System Simulator is without Additive White Gaussian Noise, and has no fading or multipath effects.

### 4.3.3	Physical channel allocations

#### 4.3.3.1	Antennas

If the UE has two Rx antennas, the same downlink signal is applied to each one. Both UE Rx antennas shall be connected unless otherwise stated in the test case.

If the UE has one Rx antenna, the downlink signal is applied to it.

#### 4.3.3.2	Downlink physical channels and physical signals

The Downlink Physical channels and Physical signals used and their relative powers are specified for single SS Tx antenna in table 4.3.3.2-1 and for two SS Tx antennas in table 4.3.3.2-2. The details of downlink power allocation for PDSCH channel are described in TS 36.213 [29] clause 5.2. For BL-UE testing, PHICH is not required and MPDCCH is required.

Table 4.3.3.2-1: Power allocation for OFDM symbols and reference signals, single SS Tx antenna

| Physical Channel | EPRE Ratio |
| --- | --- |
| PBCH | PBCH_RA = 0 dB |
|  | PBCH_RB = 0 dB |
| PSS | PSS_RA = 0 dB |
| SSS | SSS_RA = 0 dB |
| PCFICH | PCFICH_RB = 0 dB |
| PDCCH | PDCCH_RA = 0 dB |
|  | PDCCH_RB = 0 dB |
| MPDCCH | MPDCCH_RA = 0 dB |
|  | MPDCCH_RB = 0 dB |
| PDSCH | PDSCH_RA = 0 dB |
|  | PDSCH_RB = 0 dB |
| PHICH | PHICH_RB = 0 dB |
| PMCH | PMCH_RA = 0 dB |
| MBSFN RS | MBSFN RS_RA = 0dB |

Table 4.3.3.2-2: Power allocation for OFDM symbols and reference signals, two SS Tx antennas

| Physical Channel | EPRE Ratio |
| --- | --- |
| PBCH | PBCH_RA = 0 dB |
|  | PBCH_RB = 0 dB |
| PSS | PSS_RA = 0 dB |
| SSS | SSS_RA = 0 dB |
| PCFICH | PCFICH_RB = 0 dB |
| PDCCH | PDCCH_RA = 0 dB |
|  | PDCCH_RB = 0 dB |
| MPDCCH | MPDCCH_RA = 0 dB |
|  | MPDCCH_RB = 0 dB |
| PDSCH | PDSCH_RA = -3 dB |
|  | PDSCH_RB = -3 dB |
| PHICH | PHICH_RB = 0 dB |
| PMCH | PMCH_RA = 0 dB |
| MBSFN RS | MBSFN RS_RA = 0dB |

NOTE:	MBSFN RS is not defined downlink physical channels in TS 36.211 [35].

#### 4.3.3.3	Mapping of downlink physical channels and signals to physical resources

Parameters for mapping of downlink physical channels and signals are specified as follows.

Normal Cyclic Prefix

, Physical layer cell identity	= 0 is used as the default physical layer cell identity

CFI	= 3 for 1.4, 3 and 5 MHz system bandwidths

= 2 for 10, 15 and 20 MHz system bandwidths

= 2 if PMCH is configured (‘mbsfn-SubframeConfiguration’ is present in SIB2)

Ng	= 1

PHICH duration	= Normal

For Signalling testing, the default system bandwidth is 5/10/20 MHz and single SS Tx antenna is used unless specified otherwise in the test case. The mapping of downlink physical channels to physical resources for Single Tx Antenna is described in table 4.3.3.3-1 for 5/10MHz system bandwidth FDD and table 4.3.3.3-2 for 5/20MHz system bandwidth TDD.

For RF testing, the mapping of DL physical channels to resource element is defined TS 36.521-1 [21] Annex C.1.

Table 4.3.3.3-1: Mapping of DL Physical Channels to Resource Elements for Single SS Tx Antenna and 5/10 MHz System Bandwidth (FDD)

| Physical channel | Time Domain Location | Frequency Domain Location | Note |
| --- | --- | --- | --- |
| PBCH | Symbols 0 to 3 of slot 1 of subframe 0 of each radio frame | Occupies 72 subcarriers centred on the DC subcarrier | Mapping rule is specified in TS36.211 [35] sub clause 6.6.4 |
| PSS | Symbol 6 of slot 0 and 10 of each radio frame | Occupies 62 subcarriers centred on the DC subcarrier. Additional 10 subcarriers (5 on each side) adjacent to the centred 62 subcarriers are reserved. | Mapping rule is specified in TS36.211 [35] sub clause 6.11.1.2 |
| SSS | Symbol 5 of slots 0 and 10 of each radio frame | Occupies 62 subcarriers centred on the DC subcarrier. Additional 10 subcarriers (5 on each side) adjacent to the centred 62 subcarriers are reserved. | Mapping rule is specified in TS36.211 [35] sub clause 6.11.2.2 |
| PCFICH | Symbol 0 of each subframe | Maps into 4 REGs uniformly spread in the frequency domain over the whole system bandwidth. | Mapping rule is specified in TS36.211 [35] sub clause 6.7.4 - CELL_ID = 0 |
| PHICH | Symbol 0 of each subframe | Each PHICH group maps into 3 REGs in the frequency domain on the REGs not assigned to PCFICH over the whole system bandwidth, | Mapping rule is specified in TS36.211 [35] sub clause 6.9.3 - CELL_ID = 0 - Number of PHICH groups = 4(BW=5 MHz)/7(BW=10MHz) |
| PDCCH | Symbols 0, 1, 2 of each subframe (BW=5MHz) | The remaining REGs not allocated to both PCFICH and PHICH are used for PDCCH | Mapping rule is specified in TS36.211 [35] sub clause 6.8.5 - CFI = 3 (BW=5MHz) |
|  | Symbols 0, 1 of each subframe (BW=10MHz) |  | Mapping rule is specified in TS36.211 [35] sub clause 6.8.5 - CFI = 2(BW=10MHz) |
|  | Symbols 0, 1 of each MBSFN subframe |  | Mapping rule is specified in TS36.211 [35] sub clause 6.8.5 - CFI = 2 |
| MPDCCH | Same as PDSCH | For Subframe 0, subcarriers corresponding to resource elements not allocated to RS, PSS, SSS and PBCH (core set and repetitions) and reserved for PBCH antenna ports 3 and 4 on the 72 central subcarriers. For Subframe 5, subcarriers corresponding to resource elements not allocated to RS, PSS and SSS. For Subframe 9, subcarriers corresponding to resource elements not allocated to RS, PSS, SSS and PBCH (repetitions). For other subframes, subcarriers corresponding to resource elements not allocated to RS. | Mapping rule is specified in TS36.211 [35] sub clause 6.8B.5 |
| PDSCH | All remaining OFDM symbols of each subframe not allocated to PDCCH | For Subframe 0, REs not allocated to RS, PSS, SSS and PBCH is allocated to PDSCH For Subframe 5, REs not allocated to RS, PSS and SSS is allocated to PDSCH For other subframes, REs not allocated to RS is allocated to PDSCH |  |
| PMCH | MBSFN region of MBSFN subframes | REs not allocated to MBSFN RS is allocated to PMCH |  |

NOTE:	In case a single cell-specific RS is configured, cell-specific RS shall be assume to be present on antenna ports 0 and 1 for the purpose of mapping a symbol-quadruplet to a REG (resource element group). (See TS 36.211 [35] sub clause 6.2.4).

Table 4.3.3.3-2: Mapping of DL Physical Channels to Resource Elements for Single SS Tx Antenna and 5/20 MHz System Bandwidth (TDD)

| Physical channel | Time Domain Location | Frequency Domain Location | Note |
| --- | --- | --- | --- |
| PBCH | Symbols 0 to 3 of slot 1 of subframe 0 of each radio frame | Occupies 72 subcarriers centred on the DC subcarrier | Mapping rule is specified in TS36.211 [35] subclause 6.6.4 |
| PSS | Symbol 2 of slot 2 and 12 of each radio frame | Occupies 62 subcarriers centred on the DC subcarrier. Additional 10 subcarriers (5 on each side) adjacent to the centred 62 subcarriers are reserved. | Mapping rule is specified in TS36.211 [35] subclause 6.11.1.2 |
| SSS | Symbol 6 of slots 1 and 11 of each radio frame | Occupies 62 subcarriers centred on the DC subcarrier. Additional 10 subcarriers (5 on each side) adjacent to the centred 62 subcarriers are reserved. | Mapping rule is specified in TS36.211 [35] subclause 6.11.2.2 |
| PCFICH | Symbol 0 of each downlink subframe and Special subframe | Maps into 4 REGs uniformly spread in the frequency domain over the whole system bandwidth. | Mapping rule is specified in TS36.211 [35] subclause 6.7.4 - CELL_ID = 0 |
| PHICH | Symbol 0 of each downlink subframe and Special subframe | Each PHICH group maps into 3 REGs in the frequency domain on the REGs not assigned to PCFICH over the whole system bandwidth. | Mapping rule is specified in TS36.211 [35] subclause 6.9.3 - CELL_ID = 0 - Number of PHICH groups = 4(BW=5MHz)/13(BW=20MHz) |
| PDCCH | Symbols 0, 1 of subframe 1, 6 and Symbols 0, 1, 2 of other downlink subframes (BW=5MHz) | The remaining REGs not allocated to both PCFICH and PHICH are used for PDCCH | Mapping rule is specified in TS36.211 [35] subclause 6.8.5 - CFI = 3 (BW=5MHz) |
|  | Symbols 0, 1 of subframe 1, 6 and Symbols 0, 1 of other downlink subframes (BW=20MHz) |  | Mapping rule is specified in TS36.211 [35] subclause 6.8.5 - CFI = 2 (BW=20MHz) |
|  | Symbols 0, 1 of each MBSFN subframe |  | Mapping rule is specified in TS36.211 [35] sub clause 6.8.5 - CFI = 2 |
| MPDCCH | Same as PDSCH | For Subframe 0, subcarriers corresponding to resource elements not allocated to RS, SSS and PBCH (core set and repetitions) and reserved for PBCH antenna ports 3 and 4 on the 72 central subcarriers. For Subframe 5, subcarriers corresponding to resource elements not allocated to RS, SSS and PBCH (repetitions). For Subframe 1 and 6, resource elements not allocated to RS, PSS, GP and UpPTS. For other subframes, subcarriers corresponding to resource elements not allocated to RS. | Mapping rule is specified in TS36.211 [35] sub clause 6.8B.5 |
| PDSCH | All remaining OFDM symbols of each downlink subframe and DwPTS not allocated to PDCCH | For Subframe 0, REs not allocated to RS, SSS and PBCH is allocated to PDSCH For Subframe 5, REs not allocated to RS and SSS is allocated to PDSCH For Subframe 1 and 6, REs not allocated to RS, PSS, GP and UpPTS is allocated to PDSCH For other downlink subframes, REs not allocated to RS is allocated to PDSCH |  |
| PMCH | MBSFN region of MBSFN subframes | REs not allocated to MBSFN RS is allocated to PMCH |  |

NOTE 1: 	In case a single cell-specific RS is configured, cell-specific RS shall be assume to be present on antenna ports 0 and 1 for the purpose of mapping a symbol-quadruplet to a REG (resource element group). (See TS 36.211 [35] subclause 6.2.4)

NOTE 2: 	In case the default TDD configuration for subframe assignment and special subframe patterns (see subclause 4.6.3)

#### 4.3.3.4	Uplink physical channels and physical signals

[FFS].

#### 4.3.3.5	Mapping of uplink physical channels and signals to physical resources

[FFS].

### 4.3.4	Signal levels

#### 4.3.4.1	Downlink signal levels

The downlink power settings in table 4.3.4.1-1 are used unless otherwise specified in a test case.

Table 4.3.4.1-1: Default Downlink power levels

|  | Unit | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Number of RBs |  | 6 | 15 | 25 | 50 | 75 | 100 |
| Channel BW Power | dBm | -66 | -62 | -60 | -57 | -55 | -54 |
| RS EPRE | dBm/15kHz | -85 | -85 | -85 | -85 | -85 | -85 |
| Note 1: The channel bandwidth powers are informative, based on -85dBm/15kHz RS_EPRE, then scaled according to the number of RBs and rounded to the nearest integer dBm value. Full RE allocation with no boost or deboost is assumed. Note 2: The power level is specified at each UE Rx antenna. |  |  |  |  |  |  |  |

It is [FFS] whether there is a requirement to specify constant power throughout all OFDM symbols, and if so how unallocated Resource elements should be treated.

The default signal level uncertainty is +/-3dB at each test port, for any level specified. If the uncertainty value is critical for the test purpose a tighter uncertainty is specified for the related test case in TS 36.521-1 [21] Annex F or in TS 36.521-3 [34] Annex F

#### 4.3.4.2	Uplink signal levels

[FFS]

### 4.3.5	Standard test signals

#### 4.3.5.1	Downlink test signals

[FFS]

#### 4.3.5.2	Uplink test signals

[FFS]

### 4.3.6	Physical layer parameters

#### 4.3.6.1	Downlink physical layer parameters

##### 4.3.6.1.1	Physical layer parameters for DCI format 0

Default physical layer parameters for DCI format 0 are specified in table 4.3.6.1.1-1.

Table 4.3.6.1.1-1: Physical layer parameters for DCI format 0

| Parameter | Value | Value in binary |
| --- | --- | --- |
| Flag for format 0/format 1A differentiation | format 0 | "0" |
| Hopping flag | w/o Hopping | "0" |
| Resource block assignment and hopping resource allocation | Depending on test parameters | - |
| Modulation and coding scheme and redundancy version | Depending on test parameters | - |
| New data indicator | Set for every data transmission/retransmission according to the rules specified in TS 36.321 | - |
| TPC command for scheduled PUSCH | 0 dB (accumulated TPC) | "01" |
| Cyclic shift for DM RS | 0 | "000" |
| UL index (TDD only) | 2 bits as defined in sections 5.1.1.1 and 8 of TS 36.213. This field is present only for TDD configuration 0 | - |
| Downlink Assignment Index (TDD only) | 2 bits as defined in Table 7.3-X in TS 36.213. It represents the total number of PDSCH subframes with and without PDCCH and PDCCH indicating downlink SPS release in the window. This field is present only for TDD configuration 1-6 | - |
| CQI request | w/o aperiodic CQI | "0" |

##### 4.3.6.1.1A	Physical layer parameters for DCI format 0C

Default physical layer parameters for DCI format 0C are specified in table 4.3.6.1.1A-1.

Table 4.3.6.1.1A-1: Physical layer parameters for DCI format 0C

| Parameter | Value | Value in binary |
| --- | --- | --- |
| Flag for format 0C/format 1A differentiation | format 0 | "0" |
| Resource allocation type | This field is only present if . The interpretation of this field is provided in section 8.1 of TS 36.213. | - |
| Frequency Hopping flag | w/o Hopping | "0" |
| Resource block assignment | Depending on test parameters | - |
| Modulation and coding scheme | 5 bits as defined in 8.6 of TS 36.213. | - |
| Repetition number | No repetitions 3 bits as defined in section 8.0 of 36.213 | “000” |
| HARQ process number | Depending on test parameters, 3bits for FDD, 4 bits for TDD. | - |
| New data indicator | 1 bit. | - |
| Redundancy version | 2 bits. | "00" |
| TPC command for scheduled PUSCH | 0 dB (accumulated TPC) | "01" |
| Cyclic shift for DM RS and OCC index | 3 bits as defined in 5.5.2.1.1 of TS 36.211. | - |
| UL index | 2 bits as defined in 5.1.1.1, 7.2.1, 8 and 8.4 of TS 36.213. (this field is present only for TDD operation with uplink-downlink configuration 0) | - |
| Downlink Assignment Index (DAI) | 2 bits as defined in section 7.3 of TS 36.213 (This field is present only for cases with TDD primary cell and either TDD operation with uplink-downlink configurations 1-6 or FDD operation.) | - |
| CSI request | 1, 2 or 3 bits as defined in section 7.2.1 of TS 36.213. | - |
| Modulation order override | 1 bit as defined in section x.x of TS 36.213. | - |
| Precoding information | Number of bits as specified in Table 5.3.3.1.8-1 of TS 36.212 | - |

##### 4.3.6.1.1B	Physical layer parameters for DCI format 0A

Default physical layer parameters for DCI format 0A are specified in table 4.3.6.1.1B-1.

Table 4.3.6.1.1B-1: Physical layer parameters for DCI format 0A

| Parameter | Value | Value in binary |
| --- | --- | --- |
| Carrier indicator | 0 or 3 bits. This field is present according to the definitions in TS 36.213. | not present |
| Flag for format 0A/format 1A differentiation | format 0A | "0" |
| PUSCH trigger A | non-triggered scheduling | “0” |
| Timing offset | 0 | “0000” |
| Resource block assignment | Depending on test parameters | - |
| Modulation and coding scheme | Depending on test parameters | - |
| HARQ process number | Depending on test parameters | - |
| New data indicator | Set for every data transmission/retransmission according to the rules specified in TS 36.321 |  |
| Redundancy version | 2 bits. | "00" |
| TPC command for scheduled PUSCH | 0 dB (accumulated TPC) | "01" |
| Cyclic shift for DM RS and OCC index | 3 bits as defined in 5.5.2.1.1 of TS 36.211 |  |
| CSI request | w/o aperiodic CQI | “0” |
| SRS request | w/o SRS | “0” |
| PUSCH starting position | 25us in symbol 0 | “01” |
| PUSCH ending symbol | the last symbol of the subframe | “0” |
| Channel Access type | channel access Type 2 | “1” |
| Channel Access Priority Class | 1 | “01” |

##### 4.3.6.1.1C	Physical layer parameters for DCI format 0B

Default physical layer parameters for DCI format 0B are specified in table 4.3.6.1.1C-1.

Table 4.3.6.1.1C-1: Physical layer parameters for DCI format 0B

| Parameter | Value | Value in binary |
| --- | --- | --- |
| Carrier indicator | 0 or 3 bits. This field is present according to the definitions in TS 36.213. | not present |
| PUSCH trigger A | non-triggered scheduling | “0” |
| Timing offset | 0 | “0000” |
| Number of scheduled subframes | 4 | “11” |
| Resource block assignment | Depending on test parameters | - |
| Modulation and coding scheme | Depending on test parameters | - |
| HARQ process number | Depending on test parameters | - |
| New data indicator | Set for every data transmission/retransmission according to the rules specified in TS 36.321 | - |
| Redundancy version | 2 bits. | "00" |
| TPC command for scheduled PUSCH | 0 dB (accumulated TPC) | "01" |
| Cyclic shift for DM RS and OCC index | 3 bits as defined in 5.5.2.1.1 of TS 36.211 |  |
| CSI request | w/o aperiodic CQI | “0” |
| SRS request | w/o SRS | “0” |
| PUSCH starting position | 25us in symbol 0 | “01” |
| PUSCH ending symbol | the last symbol of the subframe | “0” |
| Channel Access type | channel access Type 2 | “1” |
| Channel Access Priority Class | 1 | “01” |

##### 4.3.6.1.2	Physical layer parameters for DCI format 1

Default physical layer parameters for DCI format 1 are specified in table 4.3.6.1.2-1.

Table 4.3.6.1.2-1: Physical layer parameters for DCI format 1

| Parameter | Value | Value in binary |
| --- | --- | --- |
| Resource allocation header | Resource allocation type 0 | "0" |
| Resource block assignment | Depending on test parameters | - |
| Modulation and coding scheme | Depending on test parameters | - |
| HARQ process number | Depending on test parameters, 3bits for FDD, 4 bits for TDD. | - |
| New data indicator | Set for every data transmission/retransmission according to the rules specified in TS 36.321 | - |
| Redundancy version | Depending on test parameters | "00" |
| TPC command for PUCCH | 0 dB (accumulated TPC) | "01" |
| Downlink Assignment Index (TDD only) | 2 bits as defined in Table 7.3-X in TS 36.213. It represents the number of PDSCH with PDCCH and PDCCH indicating downlink SPS release in the window up to the present subframe. This is present for all the uplink-downlink configurations and only applies to uplink -downlink configuration 1-6 | - |

##### 4.3.6.1.3	Physical layer parameters for DCI format 1A

Default physical layer parameters for DCI format 1A are specified in table 4.3.6.1.3-1.

Table 4.3.6.1.3-1: Physical layer parameters for DCI format 1A

| Parameter | Value | Value in binary |
| --- | --- | --- |
| Flag for format 0/format 1A differentiation | format 1A | "1" |
| Localized/Distributed VRB assignment flag | Localized VRB assignment | "0" |
| Resource block assignment | Depending on test parameters | - |
| Modulation and coding scheme | Depending on test parameters | - |
| HARQ process number | Depending on test parameters, 3bits for FDD, 4 bits for TDD. | - |
| New data indicator | Set for every data transmission/retransmission according to the rules specified in TS 36.321 | - |
| Redundancy version | Depending on test parameters | "00" |
| TPC command for PUCCH | 0 dB (accumulated TPC) | "01" |
| Downlink Assignment Index (TDD only) | 2 bits as defined in Table 7.3-X in TS 36.213. It represents the number of PDSCH with PDCCH and PDCCH indicating downlink SPS release in the window up to the present subframe. This is present for all the uplink-downlink configurations and only applies to uplink -downlink configuration 1-6 | - |

##### 4.3.6.1.3A	Physical layer parameters for DCI format 1B

Default physical layer parameters for DCI format 1B are specified in table 4.3.6.1.3A-1.

Table 4.3.6.1.3A-1: Physical layer parameters for DCI format 1B

| Parameter | Value | Value in binary |
| --- | --- | --- |
| Localized/Distributed VRB assignment flag | Localized VRB assignment | "0" |
| Resource block assignment | Depending on test parameters | - |
| Modulation and coding scheme | Depending on test parameters | - |
| HARQ process number | Depending on test parameters, 3bits for FDD, 4 bits for TDD. | - |
| New data indicator | Set for every data transmission/retransmission according to the rules specified in TS 36.321 | - |
| Redundancy version | Depending on test parameters | "00" |
| TPC command for PUCCH | 0 dB (accumulated TPC) | "01" |
| Downlink Assignment Index (TDD only) | 2 bits as defined in Table 7.3-X in TS 36.213. It represents the number of PDSCH with PDCCH and PDCCH indicating downlink SPS release in the window up to the present subframe. This is present for all the uplink-downlink configurations and only applies to uplink -downlink configuration 1-6 | - |
| TPMI information for precoding | Set for codebook index according to TS 36.211 Table 6.3.4.2.3-1 for 2 Tx antenna ports and Table 6.3.4.2.3-2 for 4 Tx antenna ports corresponding to the single layer transmission | - |
| PMI confirmation for precoding | Set for every transmission, 1 for precoding according to the latest reported PMI on PUSCH,0 for precoding according to the indicated TPMI in the TPMI information field | - |

##### 4.3.6.1.4	Physical layer parameters for DCI format 1C

Default physical layer parameters for DCI format 1C are specified in table 4.3.6.1.4-1.

Table 4.3.6.1.4-1: Physical layer parameters for DCI format 1C

| Parameter | Value | Value in binary |
| --- | --- | --- |
| gap value (N_RB >= 50) | gap1 | "0" |
| Resource block assignment | depending on test parameters | - |
| Transport block size index | depending on test parameters | - |

##### 4.3.6.1.5	Physical layer parameters for DCI format 2

Default physical layer parameters for DCI format 2 are specified in table 4.3.6.1.5-1.

Table 4.3.6.1.5-1: Physical layer parameters for DCI format 2

| Parameter | Value | Value in binary |
| --- | --- | --- |
| Resource allocation header | Resource allocation type 0 | "0" |
| Resource block assignment | Depending on test parameters | - |
| TPC command for PUCCH | 0 dB (accumulated TPC) | "01" |
| Downlink Assignment Index (TDD only) | 2 bits as defined in Table 7.3-X in TS 36.213. It represents the number of PDSCH with PDCCH and PDCCH indicating downlink SPS release in the window up to the present subframe. This is present for all the uplink-downlink configurations and only applies to uplink -downlink configuration 1-6 | - |
| HARQ process number | Depending on test parameters, 3bits for FDD, 4 bits for TDD. | - |
| Transport block to codeword swap flag | No swap | "0" |
| Modulation and coding scheme (transport block 1) | Depending on test parameters | - |
| New data indicator (transport block 1) | Set for every data transmission/retransmission according to the rules specified in TS 36.321 | - |
| Redundancy version (transport block 1) | Depending on test parameters | - |
| Modulation and coding scheme (transport block 2) | Depending on test parameters | - |
| New data indicator (transport block 2) | Set for every data transmission/retransmission according to the rules specified in TS 36.321 | - |
| Redundancy version (transport block 2) | Depending on test parameters | - |
| Precoding information | Set for every transmission according to 36.212 Table 5.3.3.1.5-4 for 2 Tx antenna ports and Table 5.3.3.1.5-5 for 4 Tx antenna ports | - |

##### 4.3.6.1.6	Physical layer parameters for DCI format 2A

Default physical layer parameters for DCI format 2A are specified in table 4.3.6.1.6-1.

Table 4.3.6.1.6-1: Physical layer parameters for DCI format 2A

| Parameter | Value | Value in binary |
| --- | --- | --- |
| Resource allocation header | Resource allocation type 0 | "0" |
| Resource block assignment | depending on test parameters | - |
| TPC command for PUCCH | 0 dB (accumulated TPC) | "01" |
| Downlink Assignment Index (TDD only) | 2 bits as defined in Table 7.3-X in TS 36.213. It represents the number of PDSCH with PDCCH and PDCCH indicating downlink SPS release in the window up to the present subframe. This is present for all the uplink-downlink configurations and only applies to uplink -downlink configuration 1-6 | - |
| HARQ process number | depending on test parameters, 3bits for FDD, and 4 bits for TDD. | - |
| Transport block to codeword swap flag | No swap | "0" |
| Modulation and coding scheme (transport block 1) | depending on test parameters | - |
| New data indicator (transport block 1) | Set for every data transmission/retransmission according to the rules specified in TS 36.321 | - |
| Redundancy version (transport block 1) | depending on test parameters | - |
| Modulation and coding scheme (transport block 2) | depending on test parameters | - |
| New data indicator (transport block 2) | Set for every data transmission/retransmission according to the rules specified in TS 36.321 | - |
| Redundancy version (transport block 2) | depending on test parameters | - |
| Precoding information | N/A for 2 Tx antenna ports. For 4 Tx antenna ports, set for every transmission according to 36.212 Table 5.3.3.1.5A-2 | - |

##### 4.3.6.1.6A	Physical layer parameters for DCI format 3B

Default physical layer parameters for DCI format 3B are specified in table 4.3.6.1.6A-1 for up to 5 TDD SCells configured without PUCCH/PUSCH.

Table 4.3.6.1.6A-1: Physical layer parameters for DCI format 3B for up to 5 TDD SCells configured without PUCCH/PUSCH

| Parameter | Value | Value in binary |
| --- | --- | --- |
| SRS request (block number 1) | 1 bit. The interpretation of this field is provided in section 8.2 of TS 36.213. | “1” |
| TPC command (block number 1) | 0 dB (accumulated TPC) | "01" |

##### 4.3.6.1.6B	Physical layer parameters for DCI format 4A

Default physical layer parameters for DCI format 4A are specified in table 4.3.6.1.6B-1.

Table 4.3.6.1.6B-1: Physical layer parameters for DCI format 4A for eLAA

| Parameter | Value | Value in binary |
| --- | --- | --- |
| Carrier indicator | 0 or 3 bits. This field is present according to the definitions in TS 36.213. | not present |
| PUSCH trigger A | non-triggered scheduling | “0” |
| Timing offset | 0 | “0000” |
| Resource block assignment | Depending on test parameters | - |
| HARQ process number | 2 bits. | "00" |
| Redundancy version | Depending on test parameters | - |
| TPC command for scheduled PUSCH | 0 dB (accumulated TPC) | "01" |
| Cyclic shift for DM RS and OCC index | 3 bits as defined in 5.5.2.1.1 of TS 36.211 |  |
| CSI request | w/o aperiodic CQI | “0” |
| SRS request | w/o SRS | “0” |
| PUSCH starting position | 25us in symbol 0 | “01” |
| PUSCH ending symbol | the last symbol of the subframe | “0” |
| Channel Access type | channel access Type 2 | “1” |
| Channel Access Priority Class | 1 | “01” |
| Modulation and coding scheme and redundancy version for transport block 1 | Depending on test parameters | - |
| New data indicator for transport block 1 | Depending on test parameters | - |
| Modulation and coding scheme and redundancy version for transport block 2 | Depending on test parameters | - |
| New data indicator for transport block 2 | Depending on test parameters | - |
| Precoding information and number of layers | Depending on test parameters | - |

##### 4.3.6.1.6C	Physical layer parameters for DCI format 4B

Default physical layer parameters for DCI format 4B are specified in table 4.3.6.1.6C-1.

Table 4.3.6.1.6C-1: Physical layer parameters for DCI format 4B for eLAA

| Parameter | Value | Value in binary |
| --- | --- | --- |
| Carrier indicator | 0 or 3 bits. This field is present according to the definitions in TS 36.213. | not present |
| PUSCH trigger A | non-triggered scheduling | “0” |
| Timing offset | 0 | “0000” |
| Number of scheduled subframes | 4 | “11” |
| Resource block assignment | Depending on test parameters | - |
| HARQ process number | Depending on test parameters | - |
| Redundancy version | 2 bits. | "00" |
| TPC command for scheduled PUSCH | 0 dB (accumulated TPC) | "01" |
| Cyclic shift for DM RS and OCC index | 3 bits as defined in 5.5.2.1.1 of TS 36.211 |  |
| CSI request | w/o aperiodic CQI | “0” |
| SRS request | w/o SRS | “0” |
| PUSCH starting position | 25us in symbol 0 | “01” |
| PUSCH ending symbol | the last symbol of the subframe | “0” |
| Channel Access type | channel access Type 2 | “1” |
| Channel Access Priority Class | 1 | “01” |
| Modulation and coding scheme and redundancy version for transport block 1 | Depending on test parameters | - |
| New data indicator for transport block 1 | Depending on test parameters | - |
| Modulation and coding scheme and redundancy version for transport block 2 | Depending on test parameters | - |
| New data indicator for transport block 2 | Depending on test parameters | - |
| Precoding information and number of layers | Depending on test parameters | - |

##### 4.3.6.1.7	Physical layer parameters for DCI format 5

Default physical layer parameters for DCI format 5 are specified in table 4.3.6.1.7-1.

Table 4.3.6.1.7-1: Physical layer parameters for DCI format 5

| Parameter | Value | Value in binary |
| --- | --- | --- |
| Resource for PSCCH | Depending on test parameters | - |
| TPC command for PSCCH and PSSCH |  | “0” |
| Frequency hopping flag | w/o hopping | “0” |
| Resource block assignment and hopping resource allocation |  |  |
| Time resource pattern | Depending on test parameters |  |

##### 4.3.6.1.7A	Physical layer parameters for DCI format 5A

Default physical layer parameters for DCI format 5A are specified in table 4.3.6.1.7A-1.

Table 4.3.6.1.7A-1: Physical layer parameters for DCI format 5A

| Parameter | Value | Value in binary |
| --- | --- | --- |
| Carrier indicator | Pcell’s frequency | “0” |
| Lowest index of the subchannel allocation | Depending on test parameters |  |
| Frequency resource location | Depending on test parameters |  |
| Time gap between initial transmission and retransmission | Depending on test parameters |  |

##### 4.3.6.1.8	Physical layer parameters for DCI format 6-0A

Default physical layer parameters for DCI format 6-0A for scheduling of PUSCH are specified in table 4.3.6.1.8-1.

Table 4.3.6.1.8-1: Physical layer parameters for DCI format 6-0A

| Parameter | Value | Value in binary |  |
| --- | --- | --- | --- |
| Flag format 6-0A/format 6-1A differentiation | format 0A | "0" | Condition |
| Frequency hopping flag | w/o Hopping | "0" |  |
| Resource block assignment - narrowband index | nNB = 0 according to section 5.2.4 in TS 36.211 | “0” | RF |
|  | See TS 36.523-3 clause 7.3.3.9 |  | SIG |
| Resource block assignment – UL resource allocation | Depending on test parameters 5 bits provide the resource allocation using UL resource allocation type 0 within the indicated narrowband | - |  |
| Modulation and coding scheme | Depending on test parameters | - |  |
| Repetition number | No repetitions 2 bits as defined in section 8.0 of 36.213 | “00” |  |
| HARQ process number | Depending on test parameters, 3 bits | - |  |
| New data indicator | Set for every data transmission/retransmission according to the rules specified in TS 36.321 | - |  |
| Redundancy version | Depending on test parameters | - |  |
| TPC command for scheduled PUSCH | 0 dB (accumulated TPC) | "01" |  |
| UL index (TDD only) | 2 bits as defined in sections 5.1.1.1 and 8 of TS 36.213. This field is present only for TDD configuration 0 | - |  |
| Downlink Assignment Index (TDD only) | 2 bits as defined in Table 7.3 in TS 36.213. It represents the total number of PDSCH subframes with and without PDCCH and PDCCH indicating downlink SPS release in the window. This field is present only for TDD configuration 1-6 | - |  |
| CSI request | w/o aperiodic CQI | "0" |  |
| SRS request | 1 bit. The interpretation of this field is provided in section 8.2 of TS 36.213. | “0” |  |
| DCI subframe repetition number | 0 | ”00” |  |

| Condition | Explanation |
| --- | --- |
| RF | For RF, performance and RRM testing |
| SIG | For protocol testing |

##### 4.3.6.1.9	Physical layer parameters for DCI format 6-0B

Default physical layer parameters for DCI format 6-0B are specified in table 4.3.6.1.9-1.

Table 4.3.6.1.9-1: Physical layer parameters for DCI format 6-0B

| Parameter | Value | Value in binary | Condition |
| --- | --- | --- | --- |
| Flag for format 6-0B/format 6-1B differentiation | format 0B | "0" |  |
| Resource block assignment - narrowband index | nNB = 0 according to section 5.2.4 in TS 36.211= 0 according to section 5.2.4 in TS 36.211 | “0” | RF |
|  | See TS 36.523-3 clause 7.3.3.9 |  | SIG |
| Resource block assignment – UL resource allocation | Depending on test parameters. 3 bits provide the resource allocation within the indicated narrowband as specified in section 8.1.3 of 36.213. | - |  |
| Modulation and coding scheme | Depending on test parameters | - |  |
| Repetition number | No repetitions 3 bits as defined in section 8.0 of 36.213 | “000” |  |
| HARQ process number | Depending on test parameters, 1 bit | - |  |
| New data indicator | Set for every data transmission/retransmission according to the rules specified in TS 36.321 | - |  |
| DCI subframe repetition number | 0 | ”00” |  |

| Condition | Explanation |
| --- | --- |
| RF | For RF, performance and RRM testing |
| SIG | For protocol testing |

##### 4.3.6.1.10	Physical layer parameters for DCI format 6-1A

Default physical layer parameters for DCI format 6-1A for scheduling of PDSCH are specified in table 4.3.6.1.10-1. Default physical layer parameters for DCI format 6-1A for scheduling of random access procedure are specified in table 4.3.6.1.10-2

Table 4.3.6.1.10-1: Physical layer parameters for DCI format 6-1A (PDSCH)

| Parameter | Value | Value in binary | Condition |
| --- | --- | --- | --- |
| Flag format 6-0A/format 6-1A differentiation | format 6-1A | "1" |  |
| Frequency hopping flag | w/o Hopping | "0" |  |
| Resource block assignment - narrowband index | nNB = 0 according to clause 6.2.7 in TS 36.211 | “0” | RF |
|  | See TS 36.523-3 clause 7.3.3.9 |  | SIG |
| Resource block assignment – DL resource allocation | Depending on test parameters 5 bits provide the resource allocation using DL resource allocation type 2 within the indicated narrowband | - |  |
| Modulation and coding scheme | Depending on test parameters | - |  |
| Repetition number | No repetitions 2 bits as defined in section 7.1.11 of 36.213 | “00” |  |
| HARQ process number | Depending on test parameters, 3bits for FDD, 4 bits for TDD. | - |  |
| New data indicator | Set for every data transmission/retransmission according to the rules specified in TS 36.321 | - |  |
| Redundancy version | Depending on test parameters | - |  |
| TPC command for PUCCH | 0 dB (accumulated TPC) | "01" |  |
| Downlink Assignment Index | Number of bits as specified in TS 36.212, Table 5.3.3.1.2-2. This field is reserved when the configured maximum repetition number is larger than 1 for either PDSCH or MPDCCH. | - |  |
| Antenna port(s) and scrambling identity | 2 bits indicating the values 0 to 3, as specified in TS 36.212, Table 5.3.3.1.5C-1. This field is present only if PDSCH transmission is configured with TM9. | - |  |
| SRS request | 1 bit. The interpretation of this field is provided in section 8.2 of TS 36.213. | “0” |  |
| TPMI information for precoding | Number of bits as specified in TS 36.212, Table 5.3.3.1.3A-1. Set for codebook index according to TS 36.211 Table 6.3.4.2.3-1 or Table 6.3.4.2.3-2 corresponding to the single-layer transmission. This field is present only if PDSCH transmission is configured with TM6. | - |  |
| PMI confirmation for precoding | Set for every transmission, 1 for precoding according to the latest reported PMI on PUSCH,0 for precoding according to the indicated TPMI in the TPMI information field | - |  |
| HARQ-ACK resource offset | No offset 2 bits as defined in TS 36.213, Table 10.1.2.1-1 | “0” |  |
| DCI subframe repetition number | 0 2 bits as defined in sub-clause 9.1.5 of TS 36.213 | ”00” |  |

| Condition | Explanation |
| --- | --- |
| RF | For RF, performance and RRM testing |
| SIG | For protocol testing |

Table 4.3.6.1.10-2: Physical layer parameters for DCI format 6-1A (random access)

| Parameter | Value | Value in binary | Condition |
| --- | --- | --- | --- |
| Flag format 6-0A/format 6-1A differentiation | format 6-1A | "1" |  |
| Frequency hopping flag | w/o Hopping | "0" |  |
| Resource block assignment - narrowband index | nNB = 0 according to clause 6.2.7 in TS 36.211 | “0” | RF |
|  | See TS 36.523-3 clause 7.3.3.9 |  | SIG |
| Resource block assignment – DL resource allocation | All 5 bits set to 1 | “11111” |  |
| Preamble Index | 0 | “000000” |  |
| PRACH Mask Index | All | “0000” |  |
| Starting CE level | Level 0 | “00” |  |
| NOTE: All the remaining bits in format 6-1A for compact scheduling assignment of one PDSCH codeword are set to zero |  |  |  |

| Condition | Explanation |
| --- | --- |
| RF | For RF, performance and RRM testing |
| SIG | For protocol testing |

##### 4.3.6.1.11	Physical layer parameters for DCI format 6-1B

Default physical layer parameters for DCI format 6-1B are specified in table 4.3.6.1.11-1.

Table 4.3.6.1.11-1: Physical layer parameters for DCI format 6-1B

| Parameter | Value | Value in binary | Condition |
| --- | --- | --- | --- |
| Flag for format 6-0B/format 6-1B differentiation | format 6-1B | "1" |  |
| Resource block assignment - narrowband index | nNB = 0 according to clause 6.2.7 in TS 36.211 | ”0” | RF |
|  | See TS 36.523-3 clause 7.3.3.9 |  | SIG |
| Resource block assignment - DL resource allocation | 6 PRBs | “1” |  |
| Modulation and coding scheme | Depending on test parameters | - |  |
| Repetition number | No repetitions 3 bits as defined in section 8.0 of 36.213 | “000” |  |
| HARQ process number | Depending on test parameters, 1 bit | - |  |
| New data indicator | Set for every data transmission/retransmission according to the rules specified in TS 36.321 | - |  |
| HARQ-ACK resource offset | No offset 2 bits as defined in TS 36.213, Table 10.1.2.1-1 | “0” |  |
| DCI subframe repetition number | 0– 2 bits as defined in sub-clause 9.1.5 of TS 36.213 | ”00” |  |

| Condition | Explanation |
| --- | --- |
| RF | For RF, performance and RRM testing |
| SIG | For protocol testing |

##### 4.3.6.1.12	Physical layer parameters for DCI format 6-2

Default physical layer parameters for DCI format 6-2 are specified in table 4.3.6.1.12-1 for direct indication and in Table 4.3.6.1.12-2 for paging.

Table 4.3.6.1.12-1: Physical layer parameters for DCI format 6-2 for direct Indication (Flag=0)

| Parameter | Value | Value in binary |
| --- | --- | --- |
| Flag for paging/direct indication differentiation | Direct indication | “0” |
| Direct Indication information | 8 bits provide direct indication of system information update and other fields, as defined in TS 36.331. | - |
| Reserved | Reserved information bits are added until the size is equal to that of format 6-2 with Flag=1 | - |

Table 4.3.6.1.12-2: Physical layer parameters for DCI format 6-2 for paging (Flag=1)

| Parameter | Value | Value in binary |
| --- | --- | --- |
| Flag for paging/direct indication differentiation | Paging | “1” |
| Resource block assignment | Depending on test parameters | - |
| Modulation and coding scheme | Depending on test parameters | - |
| Repetition number | {1,2,4,8} | Not configured |
| DCI subframe repetition number | 0 2 bits as defined in sub-clause 9.1.5 of TS 36.213 | ”00” |

##### 4.3.6.1.13	Physical layer parameters for DCI format 7-0A

Default physical layer parameters for DCI format 7-0A are specified in table 4.3.6.1.13-1.

Table 4.3.6.1.13-1: Physical layer parameters for DCI format 7-0A

| Parameter | Value | Value in binary |
| --- | --- | --- |
| Flag for DL/UL differentiation | 0 | "0" |
| Resource block assignment | Depending on test parameters | - |
| Modulation and coding scheme | Depending on test parameters | - |
| HARQ process number | Depending on test parameters | - |
| New data indicator | Set for every data transmission/retransmission according to the rules specified in TS 36.321 | - |
| Redundancy version | Depending on test parameters | "00" |
| TPC command for scheduled PUSCH | 0 dB (accumulated TPC) | "01" |
| DMRS pattern (only for PUSCH with subslot TTI) | 2 bits as defined in section 5.5.2.1.2 in TS 36.211 | "00" |
| Cyclic shift for DMRS and IFDMA configuration | 1 bit as defined in section 5.5.2.1.1 in TS 36.211 | - |
| UL index (TDD only) | 2 bits as defined in sections 5.1.1.1, 7.2.1, 8 and 8.4 of TS 36.213. This field is present only for TDD configuration 0 | - |
| Downlink Assignment Index (TDD only) | 2 bits as defined in Table 7.3-X in TS 36.213. It represents the total number of PDSCH slots with and without SPDCCH and SPDCCH indicating downlink SPS release in the window. This field is present only for TDD configuration 1-6 | - |
| CSI request | 1, 2 or 3 bits as defined in section 7.2.1 of TS 36.213 | - |
| SRS request (TDD only) | w/o SRS | "0" |
| Beta offset indicator (only for PUSCH with subslot TTI) | 1 bit as defined in section 8.6.3 of TS 36.213 | - |
| Cyclic Shift Field mapping table for DMRS | 1 bit as defined in section 5.5.2.1.1 of TS 36.211 | - |

##### 4.3.6.1.14	Physical layer parameters for DCI format 7-0B

Default physical layer parameters for DCI format 7-0B are specified in table 4.3.6.1.14-1.

Table 4.3.6.1.14-1: Physical layer parameters for DCI format 7-0B

| Parameter | Value | Value in binary |
| --- | --- | --- |
| Flag for DL/UL differentiation | 0 | "0" |
| Resource block assignment | Depending on test parameters | - |
| Modulation and coding scheme | Depending on test parameters | - |
| HARQ process number | Depending on test parameters | - |
| New data indicator | Set for every data transmission/retransmission according to the rules specified in TS 36.321 | - |
| Redundancy version | Depending on test parameters | "00" |
| TPC command for scheduled PUSCH | 0 dB (accumulated TPC) | "01" |
| DMRS pattern (only for PUSCH with subslot TTI) | 2 bits as defined in section 5.5.2.1.2 in TS 36.211 | "00" |
| Cyclic shift for DMRS and IFDMA configuration | 1 bit as defined in section 5.5.2.1.1 in TS 36.211 | - |
| UL index (TDD only) | 2 bits as defined in sections 5.1.1.1, 7.2.1, 8 and 8.4 of TS 36.213. This field is present only for TDD configuration 0 | - |
| Downlink Assignment Index (TDD only) | 2 bits as defined in Table 7.3-X in TS 36.213. It represents the total number of PDSCH slots with and without SPDCCH and SPDCCH indicating downlink SPS release in the window. This field is present only for TDD configuration 1-6 | - |
| CSI request | 1, 2 or 3 bits as defined in section 7.2.1 of TS 36.213 | - |
| SRS request (TDD only) | w/o SRS | "00" |
| Beta offset indicator (only for PUSCH with subslot TTI) | 1 bit as defined in section 8.6.3 of TS 36.213 | - |
| Cyclic Shift Field mapping table for DMRS | 1 bit as defined in section 5.5.2.1.1 of TS 36.211 | - |
| Precoding information and number of layers | 3 bits for transmission with 2 antenna ports and 6 bits for transmission with 4 antenna ports. The interpretation of this field is provided in Table 5.3.3.1.16-1 or Table 5.3.3.1.16-2 of TS 36.212 | - |

##### 4.3.6.1.15	Physical layer parameters for DCI format 7-1A

Default physical layer parameters for DCI format 7-1A are specified in table 4.3.6.1.15-1.

Table 4.3.6.1.15-1: Physical layer parameters for DCI format 7-1A

| Parameter | Value | Value in binary | Condition |
| --- | --- | --- | --- |
| Flag for DL/UL differentiation | 1 | "1" |  |
| Resource block assignment | Depending on test parameters | - |  |
| Modulation and coding scheme | Depending on test parameters | - |  |
| HARQ process number | Depending on test parameters | - |  |
| New data indicator | Set for every data transmission/retransmission according to the rules specified in TS 36.321 | - |  |
| Redundancy version | Depending on test parameters | "00" |  |
| TPC command for slot-SPUCCH or subslot-SPUCCH | 0 dB (accumulated TPC) | "01" |  |
| Downlink Assignment Index | 2 bits as defined in Table 7.3-X in TS 36.213. It represents the number of PDSCH with PDCCH and PDCCH indicating downlink SPS release in the window up to the present subframe. This is present for all the uplink-downlink configurations and only applies to uplink -downlink configuration 1-6 | - |  |
| Used/Unused SPDCCH resource indication | 2 bits. The interpretation of this field is provided in section 6.4.2 of TS 36.211 | - |  |
| SPUCCH resource indication | 2 bits as defined in section 10.1.2 of TS 36.213 | - |  |
| Repetition number | 2 bits as defined in Table 5.3.3.1.17-1 of TS 36.212 | - |  |

##### 4.3.6.1.16	Physical layer parameters for DCI format 7-1B

Default physical layer parameters for DCI format 7-1B are specified in table 4.3.6.1.16-1.

Table 4.3.6.1.16-1: Physical layer parameters for DCI format 7-1B

| Parameter |  | Value |  | Value in binary |  | Condition |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Flag for DL/UL differentiation |  | 1 |  | "1" |  |  |  |
| Resource block assignment |  | Depending on test parameters |  | - |  |  |  |
| Modulation and coding scheme |  | Depending on test parameters |  | - |  |  |  |
| HARQ process number |  | Depending on test parameters |  | - |  |  |  |
| New data indicator |  | Set for every data transmission/retransmission according to the rules specified in TS 36.321 |  | - |  |  |  |
| Redundancy version |  | Depending on test parameters |  | "00" |  |  |  |
| TPC command for slot-SPUCCH or subslot-SPUCCH |  | 0 dB (accumulated TPC) |  | "01" |  |  |  |
| Downlink Assignment Index |  | 2 bits as defined in Table 7.3-X in TS 36.213. It represents the number of PDSCH with PDCCH and PDCCH indicating downlink SPS release in the window up to the present subframe. This is present for all the uplink-downlink configurations and only applies to uplink -downlink configuration 1-6 |  | - |  |  |  |
| Used/Unused SPDCCH resource indication |  | 2 bits. The interpretation of this field is provided in section 6.4.2 of TS 36.211 |  | - |  |  |  |
| SPUCCH resource indication |  | 2 bits as defined in section 10.1.2 of TS 36.213 |  | - |  |  |  |
| Repetition number |  | 2 bits as defined in Table 5.3.3.1.17-1 of TS 36.212 |  | - |  |  |  |
| Precoding information |  | 1 bit for transmission with 2 antenna ports and 2 bits for transmission with 4 antenna ports. For transmission with 4 antenna ports, the precoding information field is defined according to Table 5.3.3.1.18-1 of TS 36.212 |  | - |  |  |  |

##### 4.3.6.1.17	Physical layer parameters for DCI format 7-1C

Default physical layer parameters for DCI format 7-1C are specified in table 4.3.6.1.17-1.

Table 4.3.6.1.17-1: Physical layer parameters for DCI format 7-1C

| Parameter |  | Value |  | Value in binary |  | Condition |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Flag for DL/UL differentiation |  | 1 |  | "1" |  |  |  |
| Resource block assignment |  | Depending on test parameters |  | - |  |  |  |
| Modulation and coding scheme |  | Depending on test parameters |  | - |  |  |  |
| HARQ process number |  | Depending on test parameters |  | - |  |  |  |
| New data indicator |  | Set for every data transmission/retransmission according to the rules specified in TS 36.321 |  | - |  |  |  |
| Redundancy version |  | Depending on test parameters |  | "00" |  |  |  |
| TPC command for slot-SPUCCH or subslot-SPUCCH |  | 0 dB (accumulated TPC) |  | "01" |  |  |  |
| Downlink Assignment Index |  | 2 bits as defined in Table 7.3-X in TS 36.213. It represents the number of PDSCH with PDCCH and PDCCH indicating downlink SPS release in the window up to the present subframe. This is present for all the uplink-downlink configurations and only applies to uplink -downlink configuration 1-6 |  | - |  |  |  |
| Used/Unused SPDCCH resource indication |  | 2 bits. The interpretation of this field is provided in section 6.4.2 of TS 36.211 |  | - |  |  |  |
| SPUCCH resource indication |  | 2 bits as defined in section 10.1.2 of TS 36.212 |  | - |  |  |  |
| Repetition number |  | 2 bits as defined in Table 5.3.3.1.17-1 of TS 36.212 |  | - |  |  |  |
| Precoding information |  | 4 bits for transmission with 2 antenna ports and 7 bits for transmission with 4 antenna ports. For transmission with 4 antenna ports, the precoding information field is defined according to Table 5.3.3.1.19-2 of TS 36.212 |  | - |  |  |  |

##### 4.3.6.1.18	Physical layer parameters for DCI format 7-1D

Default physical layer parameters for DCI format 7-1D are specified in table 4.3.6.1.18-1.

Table 4.3.6.1.18-1: Physical layer parameters for DCI format 7-1D

| Parameter |  | Value |  | Value in binary |  | Condition |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Flag for DL/UL differentiation |  | 1 |  | "1" |  |  |  |
| Resource block assignment |  | Depending on test parameters |  | - |  |  |  |
| Modulation and coding scheme |  | Depending on test parameters |  | - |  |  |  |
| HARQ process number |  | Depending on test parameters |  | - |  |  |  |
| New data indicator |  | Set for every data transmission/retransmission according to the rules specified in TS 36.321 |  | - |  |  |  |
| Redundancy version |  | Depending on test parameters |  | "00" |  |  |  |
| TPC command for slot-SPUCCH or subslot-SPUCCH |  | 0 dB (accumulated TPC) |  | "01" |  |  |  |
| Downlink Assignment Index |  | 2 bits as defined in Table 7.3-X in TS 36.213. It represents the number of PDSCH with PDCCH and PDCCH indicating downlink SPS release in the window up to the present subframe. This is present for all the uplink-downlink configurations and only applies to uplink -downlink configuration 1-6 |  | - |  |  |  |
| Used/Unused SPDCCH resource indication |  | 2 bits. The interpretation of this field is provided in section 6.4.2 of TS 36.211 |  | - |  |  |  |
| SPUCCH resource indication |  | 2 bits as defined in section 10.1.2 of TS 36.213 |  | - |  |  |  |
| Repetition number |  | 2 bits as defined in Table 5.3.3.1.17-1 of TS 36.212 |  | - |  |  |  |
| Precoding information |  | 3 bits for transmission with 2 antenna ports and 5 bits for transmission with 4 antenna ports. For transmission with 4 antenna ports, the precoding information field is defined according to Table 5.3.3.1.20-2 of TS 36.212 |  | - |  |  |  |

##### 4.3.6.1.19	Physical layer parameters for DCI format 7-1E

Default physical layer parameters for DCI format 7-1E are specified in table 4.3.6.1.19-1.

Table 4.3.6.1.19-1: Physical layer parameters for DCI format 7-1E

| Parameter |  | Value |  | Value in binary |  | Condition |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Flag for DL/UL differentiation |  | 1 |  | "1" |  |  |  |
| Resource block assignment |  | Depending on test parameters |  | - |  |  |  |
| Modulation and coding scheme |  | Depending on test parameters |  | - |  |  |  |
| HARQ process number |  | Depending on test parameters |  | - |  |  |  |
| New data indicator |  | Set for every data transmission/retransmission according to the rules specified in TS 36.321 |  | - |  |  |  |
| Redundancy version |  | Depending on test parameters |  | "00" |  |  |  |
| TPC command for slot-SPUCCH or subslot-SPUCCH |  | 0 dB (accumulated TPC) |  | "01" |  |  |  |
| Downlink Assignment Index |  | 2 bits as defined in Table 7.3-X in TS 36.213. It represents the number of PDSCH with PDCCH and PDCCH indicating downlink SPS release in the window up to the present subframe. This is present for all the uplink-downlink configurations and only applies to uplink -downlink configuration 1-6 |  | - |  |  |  |
| Used/Unused SPDCCH resource indication |  | 2 bits. The interpretation of this field is provided in section 6.4.2 of TS 36.211 |  | - |  |  |  |
| SPUCCH resource indication |  | 2 bits as defined in section 10.1.2 of TS 36.213 |  | - |  |  |  |
| Repetition number |  | 2 bits as defined in Table 5.3.3.1.17-1 of TS 36.212 |  | - |  |  |  |
| SRS request (TDD only) |  | 1 bit. The interpretation of this field is provided in section 8.2 of TS 36.213 |  | "0" |  |  |  |
| Scrambling indentity |  | 1 bit as defined in 6.10.3.1 of TS 38.211 |  | - |  |  |  |
| Precoding information |  | 2 bits as defined in Table 5.3.3.1.21-1 in TS 36.212. |  | "00" |  |  |  |

##### 4.3.6.1.20	Physical layer parameters for DCI format 7-1F

Default physical layer parameters for DCI format 7-1F are specified in table 4.3.6.1.20-1.

Table 4.3.6.1.20-1: Physical layer parameters for DCI format 7-1F

| Parameter |  | Value |  | Value in binary |  | Condition |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Flag for DL/UL differentiation |  | 1 |  | "1" |  |  |  |
| Resource block assignment |  | Depending on test parameters |  | - |  |  |  |
| Modulation and coding scheme |  | Depending on test parameters |  | - |  |  |  |
| HARQ process number |  | Depending on test parameters |  | - |  |  |  |
| New data indicator |  | Set for every data transmission/retransmission according to the rules specified in TS 36.321 |  | - |  |  |  |
| Redundancy version |  | Depending on test parameters |  | "00" |  |  |  |
| TPC command for slot-SPUCCH or subslot-SPUCCH |  | 0 dB (accumulated TPC) |  | "01" |  |  |  |
| Downlink Assignment Index |  | 2 bits as defined in Table 7.3-X in TS 36.213. It represents the number of PDSCH with PDCCH and PDCCH indicating downlink SPS release in the window up to the present subframe. This is present for all the uplink-downlink configurations and only applies to uplink -downlink configuration 1-6 |  | - |  |  |  |
| Used/Unused SPDCCH resource indication |  | 2 bits. The interpretation of this field is provided in section 6.4.2 of TS 36.211 |  | - |  |  |  |
| SPUCCH resource indication |  | 2 bits as defined in section 10.1.2 of TS 36.213 |  | - |  |  |  |
| Repetition number |  | 2 bits as defined in Table 5.3.3.1.17-1 of TS 36.212 |  | - |  |  |  |
| SRS request (TDD only) |  | 1 bit. The interpretation of this field is provided in section 8.2 of TS 36.213 |  | "0" |  |  |  |
| DMRS position indicator (only for PDSCH with subslot TTI) |  | 1 bit |  | "1" |  |  |  |
| Antenna port(s), scrambling identity and number of layers |  | 3 bits as specified in Table 5.3.3.1.22-1, 5.3.3.1.22-2 and 5.3.3.1.22-3 in TS 36.212. |  | "00"” |  |  |  |

##### 4.3.6.1.21	Physical layer parameters for DCI format 7-1G

Default physical layer parameters for DCI format 7-1G are specified in table 4.3.6.1.21-1.

Table 4.3.6.1.21-1: Physical layer parameters for DCI format 7-1G

| Parameter |  | Value |  | Value in binary |  | Condition |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Flag for DL/UL differentiation |  | 1 |  | "1" |  |  |  |
| Resource block assignment |  | Depending on test parameters |  | - |  |  |  |
| Modulation and coding scheme |  | Depending on test parameters |  | - |  |  |  |
| HARQ process number |  | Depending on test parameters |  | - |  |  |  |
| New data indicator |  | Set for every data transmission/retransmission according to the rules specified in TS 36.321 |  | - |  |  |  |
| Redundancy version |  | Depending on test parameters |  | "00" |  |  |  |
| TPC command for slot-SPUCCH or subslot-SPUCCH |  | 0 dB (accumulated TPC) |  | "01" |  |  |  |
| Downlink Assignment Index |  | 2 bits as defined in Table 7.3-X in TS 36.213. It represents the number of PDSCH with PDCCH and PDCCH indicating downlink SPS release in the window up to the present subframe. This is present for all the uplink-downlink configurations and only applies to uplink -downlink configuration 1-6 |  | - |  |  |  |
| Used/Unused SPDCCH resource indication |  | 2 bits. The interpretation of this field is provided in section 6.4.2 of TS 36.211 |  | - |  |  |  |
| Repetition number |  | 2 bits as defined in Table 5.3.3.1.17-1 of TS 36.212 |  | - |  |  |  |
| SRS request (TDD only) |  | 1 bit. The interpretation of this field is provided in section 8.2 of TS 36.213 |  | "0" |  |  |  |
| DMRS position indicator (only for PDSCH with subslot TTI) |  | 1 bit |  | "1" |  |  |  |
| Antenna port(s), scrambling identity and number of layers |  | 3 bits as specified in Table 5.3.3.1.22-1 in TS 36.212. |  | - |  |  |  |

## 4.4	Reference system configurations

The reference system configurations specified in this sub clause apply to all test cases unless otherwise specified.

### 4.4.1	Simulated network scenarios

The UE will eventually have to operate in either single mode networks (FDD or TDD), dual mode networks (FDD+TDD), or inter-RAT networks ((FDD or TDD) + (UTRA FDD, UTRA TDD, GSM, HRPD or 1xRTT)).

For

Simulated network scenarios to be tested are listed in this sub clause.

NOTE 1:	The number of cells specified does not necessarily correspond to the maximum number of resources to be configured simultaneously in test equipment. Please refer to Table 6.1-1 for such information.

NOTE 2: 	For NAS test cases see sub clause 6.3.2.

#### 4.4.1.1	Single cell network scenarios

For FDD and TDD basic single cell environment, Cell 1 is used.

#### 4.4.1.2	E-UTRA single mode multi cell network scenarios

For FDD or TDD basic intra-frequency multi cell environment, Cell 1, Cell 2 and Cell 4 are used.

For FDD or TDD basic inter-frequency multi cell environment, Cell 1, Cell 3 and Cell 6 are used.

For FDD or TDD basic inter-band cell environment, Cell 1 and Cell 10 are used.

For FDD or TDD multi tracking area intra-frequency multi cell environment, Cell 1 and Cell 11 are used.

For FDD or TDD multi tracking area inter-frequency multi cell environment, Cell 1 and Cell 23 are used.

For FDD or TDD multi PLMN inter-frequency multi cell environment, Cell 1, Cell 12, Cell 13 and Cell 14 are used.

#### 4.4.1.3	E-UTRA dual mode multi cell network scenarios

For FDD and TDD basic multi cell environment, Cell 1, Cell 10 and Cell 31 are used.

For FDD and TDD multi PLMN multi cell environment, Cell 1, Cell 28, Cell 29 and Cell 30 are used.

In addition, E-UTRA single mode multi cell network scenarios defined in clause 4.4.1.2 are combined with the dual mode scenarios defined in this clause when additional intra or inter-frequency cells are used.

#### 4.4.1.4	3GPP Inter-RAT network scenarios

For FDD and TDD basic inter-RAT cell environment with UTRA FDD or UTRA TDD, Cell 1 and Cell 5 are used.

For FDD and TDD inter-RAT cell environment with multi UTRA FDD or UTRA TDD cells, Cell 1 and Cell 7, Cell 8 and Cell 9 is used.

For FDD and TDD inter-RAT cell environment with GERAN, Cell 1 and Cell 24 are used.

For FDD and TDD inter-RAT cell environment with multi GERAN cells, Cell 1 and Cell 25, Cell 26 are used.

For FDD and TDD inter-RAT cell environment with (UTRA FDD or UTRA TDD) and GERAN, Cell 1, Cell 5 and Cell 24 are used.

#### 4.4.1.5	3GPP2 Inter-RAT network scenarios

For FDD and TDD inter-RAT cell environment with HRPD, Cell 1 and Cell 15 are used.

For FDD and TDD inter-RAT cell environment with multi HRPD cells, Cell 1 and Cell 15, Cell 16 and Cell 17 are used.

For FDD and TDD inter-RAT cell environment with 1xRTT, Cell 1 and Cell 19 are used.

For FDD and TDD inter-RAT cell environment with multi 1xRTT cells, Cell 1 and Cell 19, Cell 20 and Cell 21 are used.

#### 4.4.1.6	WLAN Inter-RAT network scenarios

For FDD and TDD inter-RAT cell environment with WLAN, Cell 1, Cell 27, Cell 27a and Cell 27b are used.

#### 4.4.1.7	Bluetooth Inter-RAT network scenarios

For FDD and TDD inter-RAT cell environment with Bluetooth, Cell 1, Cell 40 and Cell 41 are used.

### 4.4.2	Simulated cells

NOTE: For NAS test cases see subclause 6.3.2.

NOTE: Test frequency and range defined in table 4.4.2-1 do not apply to 36.521-1 test cases.

Test frequencies and simulated cells are defined in table 4.4.2-1. For E-UTRA cells, f1 is the default test frequency. For UTRA cells, f8 is the default test frequency. For GERAN cells, f11 is the default test frequency. For CDMA2000 HRPD cells, f14 is the default test frequency. For CDMA 2000 1xRTT cells, f17 is the default test frequency. For WLAN cells, f20 is the default test frequency.

Default parameters for simulated cells are specified in table 4.4.2-1A and table 4.4.2-2.

Common parameters for simulated cells are specified in subclauses 4.4.3 to 4.4.6A.

Other cell specific parameters are specified in subclause 4.4.7.

Table 4.4.2-1: Definition of test frequencies and simulated cells

| Test frequency | RAT | Operating band | Range | Simulated cells |
| --- | --- | --- | --- | --- |
| f1 | E-UTRA | Operating band under test | Mid (Note 1, Note 5, Note 8) | Cell 1, Cell 1a, Cell 2, Cell 4, Cell 11 (Note 4) |
| f2 | E-UTRA | Operating band under test | High (Note 1, Note 6, Note 8) | Cell 3, Cell 12, Cell 23 |
| f3 | E-UTRA | Operating band under test | Low (Note 1, Note 7, Note 8) | Cell 6, Cell 13 |
| f4 | E-UTRA | Operating band under test | (Note 1) | Cell 14 |
| f5 | E-UTRA | Operating band for inter-band cells | Mid (Note 1) | Cell 10, Cell 30, Cell 31 |
| f6 | E-UTRA | Operating band for inter-band cells | High (Note 1) | Cell 28, Cell 29, Cell 36 (Note 10) |
| f7 | E-UTRA | Operating band for inter-band cells | Low (Note 1) | Cell 35 (Note 10) |
| f8 | UTRA | Operating band for UTRA cells | Mid (Note 2) | Cell 5, Cell 7 |
| f9 | UTRA | Operating band for UTRA cells | High (Note 2) | Cell 8 |
| f10 | UTRA | Operating band for UTRA cells | Low (Note 2) | Cell 9 |
| f11 | GERAN | Operating band for GERAN cells | Mid (Note 3) | Cell 24 |
| f12 | GERAN | Operating band for GERAN cells | High (Note 3) | Cell 25 |
| f13 | GERAN | Operating band for GERAN cells | Low (Note 3) | Cell 26 |
| f14 | CDMA2000 HRPD | Operating band for CDMA2000 HRPD cells | Mid (Note 9) | Cell 15, Cell 16 |
| f15 | CDMA2000 HRPD | Operating band for CDMA2000 HRPD cells | High | Cell 17 |
| f16 | CDMA2000 HRPD | Operating band for CDMA2000 HRPD cells | Low (Note 9) | Cell 18 |
| f17 | CDMA2000 1xRTT | Operating band for CDMA2000 1xRTT cells | Mid | Cell 19, Cell 20 |
| f18 | CDMA2000 1xRTT | Operating band for CDMA2000 1xRTT cells | High | Cell 21 |
| f19 | CDMA2000 1xRTT | Operating band for CDMA2000 1xRTT cells | Low | Cell 22 |
| f20 | WLAN | Operating band for WLAN AP's | Mid | Cell 27, Cell 27a |
| f21 | E-UTRA | Operating band for Inter-band cells (third band) | Low (Note 10) | Cell 32 |
| f22 | E-UTRA | Operating band for Inter-band cells (third band) | Mid (Note 10) | Cell 33 |
| f23 | E-UTRA | Operating band for Inter-band cells (third band) | High (Note 10) | Cell 34 |
| f24 | UTRA | Operating band for Inter-RATcells (third band) | Low (Note 10) | Cell 37 |
| f25 | UTRA | Operating band for Inter-RATcells (third band) | Mid (Note 10) | Cell 38 |
| f26 | UTRA | Operating band for Inter-RATcells (third band) | High (Note 10) | Cell 39 |
| f27 | Bluetooth | Operating band for Bluetooth beacon's | Mid | Cell 40, Cell 41 |
| f28 | WLAN | Operating band for WLAN AP's | High | Cell 27b |
| Note 1: For signalling test, see clause 6.2.3. Note 2: See TS 34.108 [5] clause 5.1. Note 3: For signalling test, see clause 6.3.1.4. For RRM test, see clause 7.2.2.1. Note 4: For signalling test, simultaneous co-existence of Cell 2 with Cell 11 is not allowed. Note 5: For RRM test with intra-band contiguous CA, the set of contiguous component carriers are “Mid”, with the test frequencies specified in clauses 4.3.1.1.xA for FDD and 4.3.1.2.xA for TDD Note 6: For RRM test with intra-band contiguous CA, the set of contiguous component carriers are “High”, with the test frequencies specified in clauses 4.3.1.1.xA for FDD and 4.3.1.2.xA for TDD Note 7: For RRM test with intra-band contiguous CA, the set of contiguous component carriers are “Low”, with the test frequencies specified in clauses 4.3.1.1.xA for FDD and 4.3.1.2.xA for TDD. Note 8: For RRM test with intra-band non-contiguous CA, the test frequencies for the set of non-contiguous component carriers are specified in clauses 4.3.1.1.xA for FDD and 4.3.1.2.xA for TDD without any regard to range. Thus “Low”, “Mid” and “High” information in this table does not apply. Unless otherwise stated, test point with maximum Wgap is chosen. Note 9: For signalling test with band combination E-UTRA band 25 or EUTRA band 2 and HRPD/1xRTT band Class 1, F14 is ‘low' and F16 is ‘Mid’. Note 10: For RRM IncMon Test Cases. |  |  |  |  |

Table 4.4.2-1A: Default E-UTRA parameters for simulated cells

| cell ID | E-UTRAN Cell Identifier |  | Physical layer cell identity | rootSequenceIndex or rootSequenceIndexHighSpeed FDD | rootSequenceIndex or rootSequenceIndexHighSpeed TDD |
| --- | --- | --- | --- | --- | --- |
|  | eNB Identifier | Cell Identity |  |  |  |
| Cell 1 | '0000 0000 0000 0000 0001'B | '0000 0000'B | 0 | 22 | 0 |
| Cell 1a | '0000 0000 0000 0000 0001'B | '0000 0001'B | 1 | 22 | 0 |
| Cell 2 | '0000 0000 0000 0000 0001'B | '0000 0010'B | 2 | 86 | 8 |
| Cell 3 | '0000 0000 0000 0000 0010'B | '0000 0011'B | 3 | 22 | 0 |
| Cell 4 | '0000 0000 0000 0000 0011'B | '0000 0100'B | 4 | 150 | 16 |
| Cell 6 | '0000 0000 0000 0000 0100'B | '0000 0110'B | 6 | 22 | 0 |
| Cell 10 | '0000 0000 0000 0000 0101'B | '0000 1010'B | 10 | 22 | 0 |
| Cell 11 | '0000 0000 0000 0000 0110'B | '0000 1011'B | 11 | 214 | 24 |
| Cell 12 | '0000 0000 0000 0000 0010'B | '0000 1100'B | 12 | 86 | 8 |
| Cell 13 | '0000 0000 0000 0000 0100'B | '0000 1101'B | 13 | 86 | 8 |
| Cell 14 | '0000 0000 0000 0000 0111'B | '0000 1110'B | 14 | 22 | 0 |
| Cell 23 | '0000 0000 0000 0000 0110'B | '0001 0111'B | 23 | 150 | 16 |
| Cell 28 | '0000 0000 0000 0000 0010'B | '0001 1100'B | 28 | 86 | 8 |
| Cell 29 | '0000 0000 0000 0000 0100'B | '0001 1101'B | 29 | 86 | 8 |
| Cell 30 | '0000 0000 0000 0000 0111'B | '0001 1110'B | 30 | 86 | 8 |
| Cell 31 | '0000 0000 0000 0000 0110'B | '0001 1111'B | 31 | 214 | 24 |
| Cell 32 | '0000 0000 0000 0000 0001'B | '0010 0000'B | 32 | 22 | 0 |
| Cell 33 | '0000 0000 0000 0000 0001'B | '0010 0001'B | 33 | 22 | 0 |
| Cell 34 | '0000 0000 0000 0000 0001'B | '0010 0010'B | 34 | 22 | 0 |
| Cell 35 | '0000 0000 0000 0000 0001'B | '0010 0011'B | 35 | 22 | 0 |
| Cell 36 | '0000 0000 0000 0000 0001'B | '0010 0100'B | 36 | 22 | 0 |

Table 4.4.2-2: Default NAS parameters for simulated cells

| cell ID | Tracking Area |  |  |  | TA# list (Note 1) | GUTI (Note 2) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TA# | PLMN |  | TAC |  | MME Identifier |  | M-TMSI |
|  |  | MCC | MNC |  |  | MME Group ID | MME Code |  |
| Cell 1 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 32769 | 1 | Arbitrarily selected according to TS 23.003 subclause 2.8 [2]. |
| Cell 1a | TAI-1 | (Note 3) |  | 1 | TAI-1 | 32769 | 1 |  |
| Cell 2 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 32769 | 1 |  |
| Cell 3 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 32769 | 1 |  |
| Cell 4 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 32769 | 1 |  |
| Cell 6 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 32769 | 1 |  |
| Cell 10 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 32769 | 1 |  |
| Cell 11 | TAI-2 | (Note 3) |  | 2 | TAI-2 | 32769 | 1 |  |
| Cell 23 | TAI-2 | (Note 3) |  | 2 | TAI-2 | 32769 | 1 |  |
| Cell 12, Cell 28 | TAI-3 | 002 | 11 | 1 | TAI-3 | 32769 | 1 |  |
| Cell 13, Cell 29 | TAI-4 | 003 | 21 | 1 | TAI-4 | 32769 | 1 |  |
| Cell 14, Cell 30 | TAI-5 | 004 | 31 | 1 | TAI-5 | 32769 | 1 |  |
| Cell 31 | TAI-2 | (Note 3) |  | 2 | TAI-2 | 32769 | 1 |  |
| Cell 32 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 32769 | 1 |  |
| Cell 33 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 32769 | 1 |  |
| Cell 34 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 32769 | 1 |  |
| Cell 35 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 32769 | 1 |  |
| Cell 36 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 32769 | 1 |  |
| Note 1: The value(s) in the column TA# list indicates TAI(s) included in the response messages of the registration procedure (ATTACH ACCEPT or TRACKING AREA UPDATE ACCEPT) when the UE performs the registration procedure on a corresponding cell. Note 2: The value in the column GUTI indicates GUTI included in the response messages of the registration procedure (ATTACH ACCEPT or TRACKING AREA UPDATE ACCEPT) when the UE performs the registration procedure on a corresponding cell. Note 3: Set to the same Mobile Country Code and Mobile Network Code stored in EFIMSI on the test USIM card (subclause 4.9.3). |  |  |  |  |  |  |  |  |

Table 4.4.2-3: Default HRPD parameters for simulated cells

| Cell ID | SectorID | ColorCode | Pilot PN Offset | CountryCode |
| --- | --- | --- | --- | --- |
| Cell 15 | Set according to PIXIT parameter | Set according to PIXIT parameter | 50 | (Note 1) |
| Cell 16 | Set according to PIXIT parameter | Set according to PIXIT parameter | 100 | (Note 1) |
| Cell 17 | Set according to PIXIT parameter | Set according to PIXIT parameter | 150 | (Note 1) |
| Cell 18 | Set according to PIXIT parameter | Set according to PIXIT parameter | 200 | (Note 1) |
| Note 1: Set to the same Mobile Country Code stored in EFIMSI on the test USIM card (subclause 4.9.3). |  |  |  |  |

Table 4.4.2-4: Default 1XRTT parameters for simulated cells

| Cell ID | SID | NID | BASE_ID | Pilot PN Offset | REG_ZONE | MCC | IMSI_11_12 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Cell 19 | 200 | Set according to PIXIT parameter | Set according to PIXIT parameter | 50 | 1 | Note 1 | Note 2 |
| Cell 20 | 200 | Set according to PIXIT parameter | Set according to PIXIT parameter | 100 | 1 | Note 1 | Note 2 |
| Cell 21 | 200 | Set according to PIXIT parameter | Set according to PIXIT parameter | 150 | 2 | Note 1 | Note 2 |
| Cell 22 | 200 | Set according to PIXIT parameter | Set according to PIXIT parameter | 200 | 3 | Note 1 | Note 2 |
| Note 1: Set to the same Mobile Country Code stored in EFIMSI on the test USIM card (subclause 4.9.3). Note 2: Set to the same Mobile Network Code stored in EFIMSI on the test USIM card (subclause 4.9.3). |  |  |  |  |  |  |  |

Table 4.4.2-5: Neighbouring cell list for 1XRTT cells

| Cell ID | Intra Frequency | Inter Frequency Cells |  |  |
| --- | --- | --- | --- | --- |
| Cell 19 | cell 20 | Cell 21 | cell 22 | X |
| Cell 20 | cell 19 | Cell 21 | cell 22 | X |
| Cell 21 | X | cell 19 | cell 20 | cell 22 |
| Cell 22 | X | cell 19 | cell 20 | cell 21 |

### 4.4.3	Common parameters for simulated E-UTRA cells

The parameters specified in this sub clause apply to all simulated E-UTRA cells unless otherwise specified.

#### 4.4.3.1	Common configurations of system information blocks

For BL/CE testing the same content of system information blocks SIB2 to SIB20 are used for broadcasting on BCCH and/or BCCH-BR.

##### 4.4.3.1.1	Combinations of system information blocks

The combination of system information blocks required by a test case depends on the test case scenario. In this clause, the following combinations of system information blocks are defined.

Combination 1 is the default combination which applies to the following test case scenarios:

- E-UTRA FDD single cell scenario

- E-UTRA TDD single cell scenario

- E-UTRA FDD intra-frequency multi cell scenario

- E-UTRA TDD intra-frequency multi cell scenario

- E-UTRA FDD and E-UTRA TDD dual mode multi cell roaming scenario

- 3GPP inter-RAT E-UTRA FDD + NR FDD multi cell scenario for RRM

- 3GPP inter-RAT E-UTRA FDD + NR TDD multi cell scenario for RRM

- 3GPP inter-RAT E-UTRA TDD + NR FDD multi cell scenario for RRM

- 3GPP inter-RAT E-UTRA TDD + NR TDD multi cell scenario for RRM

Combination 2 applies to the following test case scenarios:

- E-UTRA FDD intra-frequency multi cell scenario with neighbouring cell related information

- E-UTRA TDD intra-frequency multi cell scenario with neighbouring cell related information

Combination 3 applies to the following test case scenarios:

- E-UTRA FDD inter-frequency multi cell scenario

- E-UTRA TDD inter-frequency multi cell scenario

- E-UTRA FDD inter-band multi cell scenario

- E-UTRA TDD inter-band multi cell scenario

- E-UTRA FDD and E-UTRA TDD dual mode multi cell non-roaming scenario

- E-UTRA FDD intra-band carrier aggregation component carriers cell scenario

- E-UTRA FDD inter-band carrier aggregation component carriers cell scenario

- E-UTRA TDD intra-band carrier aggregation component carriers cell scenario

- E-UTRA FDD and E-UTRA TDD inter-band carrier aggregation component carriers cell scenario

- E-UTRA FDD dual connectivity cell scenario

- E-UTRA TDD dual connectivity cell scenario

Combination 4 applies to the following test case scenarios:

- 3GPP inter-RAT E-UTRA FDD + UTRA FDD multi cell scenario

- 3GPP inter-RAT E-UTRA TDD + UTRA LCR TDD multi cell scenario

- 3GPP inter-RAT E-UTRA TDD + UTRA FDD multi cell scenario

Combination 5 applies to the following test case scenarios:

- 3GPP inter-RAT E-UTRA FDD + GERAN multi cell scenario

- 3GPP inter-RAT E-UTRA TDD + GERAN multi cell scenario

Combination 6 applies to the following test case scenarios:

- 3GPP2 inter-RAT E-UTRA FDD + HRPD multi cell scenario

- 3GPP2 inter-RAT E-UTRA TDD + HRPD multi cell scenario

- 3GPP2 inter-RAT E-UTRA FDD + 1xRTT multi cell scenario

- 3GPP2 inter-RAT E-UTRA TDD + 1xRTT multi cell scenario

Combination 7 applies to the following test case scenarios:

- E-UTRA FDD + home eNB multi cell scenario

- E-UTRA TDD + home eNB multi cell scenario

Combination 8 applies to the following test case scenarios:

- E-UTRA FDD ETWS single cell scenario

- E-UTRA TDD ETWS single cell scenario

Combination 9 applies to the following test case scenarios:

- E-UTRA FDD inter-frequency + 3GPP inter-RAT UTRA multi-cell scenario

- E-UTRA TDD inter-frequency + 3GPP inter-RAT UTRA multi-cell scenario

Combination 10 applies to the following test case scenarios:

- 3GPP inter-RAT E-UTRA FDD + UTRA FDD + GERAN multi cell scenario

- 3GPP inter-RAT E-UTRA TDD + UTRA LCR TDD + GERAN multi cell scenario.

Combination 10a applies to the following test case scenarios:

- E-UTRA FDD inter-frequency + 3GPP inter-RAT E-UTRA FDD + UTRA FDD + GERAN multi cell scenario

- E-UTRA TDD inter-frequency + 3GPP inter-RAT E-UTRA TDD + UTRA LCR TDD + GERAN multi cell scenario

Combination 11 applies to the following test case scenarios:

- 3GPP inter-RAT E-UTRA FDD + UTRA FDD + home eNB multi cell scenario

- 3GPP inter-RAT E-UTRA TDD + UTRA LCR TDD + home eNB multi cell scenario

Combination 12 applies to the following test case scenarios:

- E-UTRA FDD inter-frequency + GERAN cell scenario

- E-UTRA TDD inter-frequency + GERAN cell scenario

Combination 13 applies to the following test case scenarios:

- E-UTRA FDD inter-frequency + home eNB

- E-UTRA TDD inter-frequency + home eNB

Combination 14 applies to the following test case scenarios:

- 3GPP inter-RAT E-UTRA FDD + GERAN + home eNB multi cell scenario

- 3GPP inter-RAT E-UTRA TDD + GERAN + home eNB multi cell scenario

Combination 15 applies to the following test case scenarios:

- 3GPP E-UTRA FDD single cell scenario + MBMS

- 3GPP E-UTRA TDD single cell scenario + MBMS

- 3GPP E-UTRA FDD intra-frequency multi cell scenario + MBMS

- 3GPP E-UTRA TDD intra-frequency multi cell scenario + MBMS

Combination 16 applies to the following test case scenarios:

- 3GPP E-UTRA FDD inter-frequency multi cell scenario + MBMS

- 3GPP E-UTRA TDD inter-frequency multi cell scenario + MBMS

Combination 17 applies to the following test case scenarios:

- 3GPP E-UTRA FDD + CMAS single cell scenario

- 3GPP E-UTRA TDD + CMAS single cell scenario

Combination 18 applies to the following test case scenarios:

- 3GPP E-UTRA FDD + MBMS inter-frequency multi cell scenario (non-MBMS cell broadcasting SIB15)

- 3GPP E-UTRA TDD + MBMS inter-frequency multi cell scenario (non-MBMS cell broadcasting SIB15)

- 3GPP E-UTRA FDD + SC-PTM inter-frequency multi cell scenario (non- SC-PTM cell broadcasting SIB15)

- 3GPP E-UTRA TDD + SC-PTM inter-frequency multi cell scenario (non- SC-PTM cell broadcasting SIB15)

Combination 19 applies to the following test case scenarios:

- 3GPP E-UTRA FDD + MBMS intra-frequency multi cell scenario (MBMS cell broadcasting SIB15)

- 3GPP E-UTRA TDD + MBMS intra-frequency multi cell scenario (MBMS cell broadcasting SIB15)

Combination 20 applies to the following test case scenarios:

- 3GPP E-UTRA FDD + MBMS inter-frequency multi cell scenario (MBMS cell broadcasting SIB15)

- 3GPP E-UTRA TDD + MBMS inter-frequency multi cell scenario (MBMS cell broadcasting SIB15)

- 3GPP E-UTRA FDD + MBMS inter-band multi cell scenario (MBMS cell broadcasting SIB15)

- 3GPP E-UTRA TDD + MBMS inter-band multi cell scenario (MBMS cell broadcasting SIB15)

Combination 21 applies to the following test case scenarios:

- E-UTRA FDD SIMTC single cell scenario

- E-UTRA TDD SIMTC single cell scenario

Combination 22 applies to the following test case scenarios:

- RAN assisted WLAN offload: E-UTRA FDD + WLAN AP scenarios

- RAN assisted WLAN offload: E-UTRA TDD + WLAN AP scenarios

Combination 23 is the default combination which applies to the following test case scenarios:

- E-UTRA FDD single cell scenario + sidelink direct communication

- E-UTRA TDD single cell scenario + sidelink direct communication

- E-UTRA FDD intra-frequency multi cell scenario + sidelink direct communication

- E-UTRA TDD intra-frequency multi cell scenario + sidelink direct communication

Combination 24 is the default combination which applies to the following test case scenarios:

- E-UTRA FDD single cell scenario + sidelink direct discovery

- E-UTRA TDD single cell scenario + sidelink direct discovery

- E-UTRA FDD intra-frequency multi cell scenario + sidelink direct discovery

- E-UTRA TDD intra-frequency multi cell scenario + sidelink direct discovery

Combination 25 applies to the following test case scenarios:

- 3GPP E-UTRA FDD single cell scenario + SC-PTM

- 3GPP E-UTRA TDD single cell scenario + SC-PTM

- 3GPP E-UTRA FDD intra-frequency multi cell scenario + SC-PTM

- 3GPP E-UTRA TDD intra-frequency multi cell scenario + SC-PTM

Combination 26 applies to the following test case scenarios:

- 3GPP E-UTRA FDD inter-frequency multi cell scenario + SC-PTM

- 3GPP E-UTRA TDD inter-frequency multi cell scenario + SC-PTM

Combination 27 applies to the following test case scenarios:

- 3GPP E-UTRA FDD + SC-PTM intra-frequency multi cell scenario (SC-PTM cell broadcasting SIB15)

- 3GPP E-UTRA TDD + SC-PTM intra-frequency multi cell scenario (SC-PTM cell broadcasting SIB15)

Combination 28 applies to the following test case scenarios:

- 3GPP E-UTRA FDD + SC-PTM inter-frequency multi cell scenario (SC-PTM cell broadcasting SIB15)

- 3GPP E-UTRA TDD + SC-PTM inter-frequency multi cell scenario (SC-PTM cell broadcasting SIB15)

- 3GPP E-UTRA FDD + SC-PTM inter-band multi cell scenario (SC-PTM cell broadcasting SIB15)

- 3GPP E-UTRA TDD + SC-PTM inter-band multi cell scenario (SC-PTM cell broadcasting SIB15)

Combination 29 is the default combination which applies to the following test case scenarios:

- E-UTRA FDD single cell scenario + V2X sidelink communication.

- E-UTRA TDD single cell scenario + V2X sidelink communication.

- E-UTRA FDD intra-frequency multi cell scenario + V2X sidelink communication.

- E-UTRA TDD intra-frequency multi cell scenario + V2X sidelink communication.

- E-UTRA FDD inter-frequency + V2X sidelink communication with inter-frequency scheduling

- E-UTRA TDD inter-frequency + V2X sidelink communication with inter-frequency scheduling

- E-UTRA FDD inter-band + V2X sidelink communication with inter-frequency scheduling

- E-UTRA TDD inter-band + V2X sidelink communication with inter-frequency scheduling

Combination 30 applies to the following test case scenarios:

- E-UTRA FDD intra-band carrier aggregation component carriers cell scenario + E-UTRA FDD intra-frequency neighbour.

- E-UTRA FDD inter-band carrier aggregation component carriers cell scenario+ E-UTRA FDD intra-frequency neighbour.

- E-UTRA TDD intra-band carrier aggregation component carriers cell scenario+ E-UTRA FDD intra-frequency neighbour.

- E-UTRA FDD and E-UTRA TDD inter-band carrier aggregation component carriers cell scenario+ E-UTRA FDD intra-frequency neighbour.

Combination 31 applies to the following test case scenarios:

-	3GPP inter-RAT E-UTRA FDD + NR FDD multi cell scenario

-	3GPP inter-RAT E-UTRA FDD + NR TDD multi cell scenario

-	3GPP inter-RAT E-UTRA TDD + NR FDD multi cell scenario

-	3GPP inter-RAT E-UTRA TDD + NR TDD multi cell scenario

Table 4.4.3.1.1-1: Combinations of system information blocks

|  | System information block type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Combination No. | SIB2 | SIB3 | SIB4 | SIB5 | SIB6 | SIB7 | SIB8 | SIB9 | SIB10 | SIB11 | SIB12 | SIB13 | SIB 14 | SIB 15 | SIB 17 | SIB 18 | SIB 19 | SIB 20 | SIB21 | SIB24 |
| 1 | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | X | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | X | X |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | X | X |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 | X | X |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 | X | X |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 | X | X | X |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 | X | X |  |  |  |  |  |  | X | X |  |  |  |  |  |  |  |  |  |  |
| 9 | X | X |  | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 | X | X |  |  | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10a | X | X |  | X | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11 | X | X | X |  | X |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 | X | X |  | X |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | X | X | X | X |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | X | X | X |  |  | X |  | X |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | X | X |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |
| 16 | X | X |  | X |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |
| 17 | X | X |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |
| 18 | X | X |  | X |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| 19 | X | X |  |  |  |  |  |  |  |  |  | X |  | X |  |  |  |  |  |  |
| 20 | X | X |  | X |  |  |  |  |  |  |  | X |  | X |  |  |  |  |  |  |
| 21 | X | X |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |
| 22 | X | X |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |
| 23 | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |
| 24 | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |
| 25 | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |
| 26 | X | X |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |
| 27 | X | X |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  | X |  |  |
| 28 | X | X |  | X |  |  |  |  |  |  |  |  |  | X |  |  |  | X |  |  |
| 29 | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |
| 30 | X | X | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 31 | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |

##### 4.4.3.1.2	Scheduling of system information blocks

The scheduling configurations for combinations of system information blocks are defined in the following tables.

Table 4.4.3.1.2-1: Scheduling for combination 1

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |

Table 4.4.3.1.2-2: Scheduling for combination 2

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB4 |

Table 4.4.3.1.2-3: Scheduling for combination 3

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB5 |

Table 4.4.3.1.2-4: Scheduling for combination 4

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB6 |

Table 4.4.3.1.2-5: Scheduling for combination 5

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB7 |

Table 4.4.3.1.2-6: Scheduling for combination 6

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB8 |

Table 4.4.3.1.2-7: Scheduling for combination 7

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB4 |
| 4 | See sub clause 4.4.3.4 | SIB9 |

Table 4.4.3.1.2-8: Scheduling for combination 8

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB11 |
| 4 | See sub clause 4.4.3.4 | SIB10 |

Table 4.4.3.1.2-9: Scheduling for combination 9

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB5, SIB6 |

Table 4.4.3.1.2-10: Scheduling for combination 10

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB6 |
| 4 | See sub clause 4.4.3.4 | SIB7 |

Table 4.4.3.1.2-10a: Scheduling for combination 10a

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB5, SIB6 |
| 4 | See sub clause 4.4.3.4 | SIB7 |

Table 4.4.3.1.2-11: Scheduling for combination 11

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB4 |
| 4 | See sub clause 4.4.3.4 | SIB6 |
| 5 | See sub clause 4.4.3.4 | SIB9 |

Table 4.4.3.1.2-12: Scheduling for combination 12

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB5 |
| 4 | See sub clause 4.4.3.4 | SIB7 |

Table 4.4.3.1.2-13: Scheduling for combination 13

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB4 |
| 4 | See sub clause 4.4.3.4 | SIB5 |

Table 4.4.3.1.2-14: Scheduling for combination 14

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB4 |
| 4 | See sub clause 4.4.3.4 | SIB7 |
| 5 | See sub clause 4.4.3.4 | SIB9 |

Table 4.4.3.1.2-15: Scheduling for combination 15

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB13 |

Table 4.4.3.1.2-16: Scheduling for combination 16

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB5 |
| 4 | See sub clause 4.4.3.4 | SIB13 |

Table 4.4.3.1.2-17: Scheduling for combination 17

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB12 |

Table 4.4.3.1.2-18: Scheduling for combination 18

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB5 |
| 4 | See sub clause 4.4.3.4 | SIB15 |

Table 4.4.3.1.2-19: Scheduling for combination 19

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB13 |
| 4 | See sub clause 4.4.3.4 | SIB15 |

Table 4.4.3.1.2-20: Scheduling for combination 20

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB5 |
| 4 | See sub clause 4.4.3.4 | SIB13 |
| 5 | See sub clause 4.4.3.4 | SIB15 |

Table 4.4.3.1.2-21: Scheduling for combination 21

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB14 |

Table 4.4.3.1.2-22: Scheduling for combination 22

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB17 |

Table 4.4.3.1.2-23: Scheduling for combination 23

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB18 |

Table 4.4.3.1.2-24: Scheduling for combination 24

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB19 |

Table 4.4.3.1.2-25: Scheduling for combination 25

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB20 |

Table 4.4.3.1.2-26: Scheduling for combination 26

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB5 |
| 4 | See sub clause 4.4.3.4 | SIB20 |

Table 4.4.3.1.2-27: Scheduling for combination 27

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB15 |
| 4 | See sub clause 4.4.3.4 | SIB20 |

Table 4.4.3.1.2-28: Scheduling for combination 28

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB5 |
| 4 | See sub clause 4.4.3.4 | SIB15 |
| 5 | See sub clause 4.4.3.4 | SIB20 |

Table 4.4.3.1.2-29: Scheduling for combination 29

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB21 |

Table 4.4.3.1.2-30: Scheduling for combination 30

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB4 |
| 4 | See sub clause 4.4.3.4 | SIB5 |

Table 4.4.3.1.2-31: Scheduling for combination 31

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 16 | SIB2 |
| 2 | See sub clause 4.4.3.4 | SIB3 |
| 3 | See sub clause 4.4.3.4 | SIB24 |

#### 4.4.3.2	Common contents of system information messages

#### -	MasterInformationBlock

The MasterInformationBlock includes the system information transmitted on BCH.

Table 4.4.3.2-1: MasterInformationBlock

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MasterInformationBlock ::= SEQUENCE { |  |  |  |
| dl-Bandwidth | Downlink system bandwidth under test. |  |  |
| phich-Config SEQUENCE {} | PHICH-Config-DEFAULT | See subclause 4.6.3 |  |
| systemFrameNumber | A valid value as defined in TS 36.331 [17] |  |  |
| schedulingInfoSIB1-BR-r13 | 0 | Value 0 means that SystemInformationBlockType1-BR is not scheduled. |  |
| schedulingInfoSIB1-BR-r13 | 10 | 4 PDSCH repetitions and 504 bits TBS carrying for PDSCH carrying SystemInformationBlockType1-BR (TS 36.213 [23] Table 7.1.6.1 and Table 7.1.7.2.7-1) | CEmodeA, CEmodeB |
| spare | '0000 0000 00'B |  |  |
| spare | '0000 0'B |  | CEmodeA, CEmodeB |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| CEmodeA | CE mode A test environment |
| CEmodeB | CE mode B test environment |

#### -	SystemInformation

The SystemInformation message is used to convey one or more System Information Blocks. All the SIBs included are transmitted with the same periodicity.

Table 4.4.3.2-2: SystemInformation

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| systemInformation-r8 SEQUENCE { |  |  |  |
| sib-TypeAndInfo SEQUENCE (SIZE (1..maxSIB)) OF CHOICE {} | See subclause 4.4.3.1 |  |  |
| criticalExtensionsFuture SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### -	SystemInformation-BR-r13

The SystemInformation-BR-r13 message is used to convey one or more System Information Blocks for Bandwith Redused cell envirionment . All the SIBs included are transmitted with the same periodicity.

Table 4.4.3.2-2A: SystemInformation-BR-r13

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformation-BR-r13 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| systemInformation-r8 SEQUENCE { |  |  |  |
| sib-TypeAndInfo SEQUENCE (SIZE (1..maxSIB)) OF CHOICE {} | See subclause 4.4.3.1 |  |  |
| criticalExtensionsFuture SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### -	SystemInformationBlockType1

SystemInformationBlockType1 contains information relevant when evaluating if a UE is allowed to access a cell and defines the scheduling of other system information.

Table 4.4.3.2-3: SystemInformationBlockType1

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { | 1 entry |  |  |
| plmn-Identity[1] SEQUENCE { |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit | See table 4.4.2-2 | For NAS test cases, see table 6.3.2.2-1. |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit | See table 4.4.2-2 | For NAS test cases, see table 6.3.2.2-1. |  |
| } |  |  |  |
| cellReservedForOperatorUse[1] | notReserved |  |  |
| } |  |  |  |
| trackingAreaCode | See table 4.4.2-2 | For NAS test cases, see table 6.3.2.2-1. |  |
| cellIdentity | Cell ID for the simulated cell |  |  |
| cellBarred | notBarred |  |  |
| intraFreqReselection | notAllowed |  |  |
| csg-Indication | FALSE |  |  |
| csg-Identity | Not present |  |  |
| } |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -70 (-140 dBm) | For RF/RRM test cases | RF |
|  | -53 (-106 dBm) | For signalling test cases in normal coverage |  |
|  | -27 (-54dBm) | For signalling test cases in enhanced coverage The value has been selected such that the UE at signal levels for ”Serving cell” and “Suitable neighbour intra-frequency cell” are applied then the cell selection criterion for normal coverage is not fullfilled. | SIG AND ( CEmodeA OR CEmodeB) |
| q-RxLevMinOffset | Not present |  |  |
| } |  |  |  |
| p-Max | Not present |  |  |
| freqBandIndicator | Operating band under test. |  |  |
| freqBandIndicator | 64 | maxFBI | Band > 64 |
| schedulingInfoList SEQUENCE (SIZE (1..maxSI-Message)) OF SEQUENCE {} | See subclause 4.4.3.1 |  |  |
| tdd-Config SEQUENCE {} | Not present |  | FDD |
| tdd-Config SEQUENCE {} | TDD-Config-DEFAULT | See subclause 4.6.3 | TDD |
| si-WindowLength | ms20 | To allow sufficient number of retransmissions. |  |
| systemInfoValueTag | 0 |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension SEQUENCE { | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  | Band > 64 |
| freqBandIndicator-v9e0 | Operating band under test. |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ims-EmergencySupport-r9 | True | Support IMS emergency call in limited service mode. |  |
| cellSelectionInfo-v920 SEQUENCE {} | Not present |  |  |
| cellSelectionInfo-v920 SEQUENCE { |  |  | QBASED |  |
| q-QualMin-r9 | -20 (-20dB) |  |  |  |
| q-QualMinOffset-r9 | Not present |  |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { | Not present |  |  |
| tdd-Config-v1130 | Not present |  |  |  |  |
| cellSelectionInfo-v1130 | Not present |  |  |  |  |
| nonCriticalExtension | Not present |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | UECAT0 |  |  |
| cellAccessRelatedInfo-v1250 SEQUENCE { |  |  |  |  |  |
| category0Allowed-r12 | True | Cat 0 Allowed |  |  |  |
| } |  |  |  |  |  |
| cellSelectionInfo-v1250 | Not present |  |  |  |  |
| freqBandIndicatorPriority-r12 | Not present |  |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |  |  |
| hyperSFN-r13 | Not present |  |  |  |  |
| eDRXAllowed-r13 | Not present |  |  |  |  |
| cellSelectionInfoCE-r13 | Not present |  |  |  |  |
| bandwidthReducedAccessRelatedInfo-r13 | Not present |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | eCalloverIMS |  |  |
| eCallOverIMS-Support-r14 | True | Support eCall over IMS services. |  |  |  |
| tdd-Config-v1430 | Not present |  |  |  |  |
| cellAccessRelatedInfoList-r14 SEQUENCE { } | Not present |  |  |  |  |
| nonCriticalExtension | Not present |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |
| QBASED | This condition applies to Quality based cell (re)selection signalling test cases. |
| UECAT0 | This condition applies when UE under test is a UE of Category 0. |
| RF | For RF, performance and RRM testing |
| SIG | For protocol testing |
| CEmodeA | CE mode A test environment |
| CEmodeB | CE mode B test environment |
| Band > 64 | If band > 64 is selected |
| eCalloverIMS | eCall over IMS test environment |

#### -	SystemInformationBlockType1-BR-r13

SystemInformationBlockType1-BR-r13 contains information relevant when evaluating if a UE in a Bandwith Reduced cell environment is allowed to access a cell and defines the scheduling of other system information.

Table 4.4.3.2-3A: SystemInformationBlockType1-BR-r13

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF SEQUENCE { | 1 entry |  |  |
| plmn-Identity[1] SEQUENCE { |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit | See table 4.4.2-2 | For NAS test cases, see table 6.3.2.2-1. |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit | See table 4.4.2-2 | For NAS test cases, see table 6.3.2.2-1. |  |
| } |  |  |  |
| cellReservedForOperatorUse[1] | notReserved |  |  |
| } |  |  |  |
| trackingAreaCode | See table 4.4.2-2 | For NAS test cases, see table 6.3.2.2-1. |  |
| cellIdentity | Cell ID for the simulated cell |  |  |
| cellBarred | notBarred |  |  |
| intraFreqReselection | notAllowed |  |  |
| csg-Indication | FALSE |  |  |
| csg-Identity | Not present |  |  |
| } |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -70 (-140 dBm) | For RF/RRM test cases | RF |
|  | -27 (-54 dBm) | For signalling test cases | SIG and ENHANCED COVERAGE |
|  | -53 (-106 dBm) | For signalling test cases | SIG and NORMAL COVERAGE |
| q-RxLevMinOffset | Not present |  |  |
| } |  |  |  |
| p-Max | Not present |  |  |
| freqBandIndicator | Operating band under test. |  |  |
| freqBandIndicator | 64 | maxFBI | Band > 64 |
| schedulingInfoList SEQUENCE (SIZE (1..maxSI-Message)) OF SEQUENCE {} | See subclause 4.4.3.1 |  |  |
| tdd-Config SEQUENCE {} | Not present |  | FDD |
| tdd-Config SEQUENCE {} | TDD-Config-DEFAULT | See subclause 4.6.3 | TDD |
| si-WindowLength | ms20 | To allow sufficient number of retransmissions. |  |
| systemInfoValueTag | 0 |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  | Band > 64 |
| freqBandIndicator-v9e0 | Operating band under test. |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ims-EmergencySupport-r9 | True | Support IMS emergency call in limited service mode. |  |
| cellSelectionInfo-v920 SEQUENCE {} | Not present |  |  |
| cellSelectionInfo-v920 SEQUENCE { |  |  | QBASED |
| q-QualMin-r9 | -20 (-20dB) |  |  |
| q-QualMinOffset-r9 | Not present |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| tdd-Config-v1130 | Not present |  |  |
| cellSelectionInfo-v1130 | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| cellAccessRelatedInfo-v1250 SEQUENCE { |  |  |  |
| category0Allowed-r12 | Not Present |  |  |
| category0Allowed-r12 | True | Cat 0 Allowed | UECAT0 |
| } |  |  |  |
| cellSelectionInfo-v1250 | Not present |  |  |
| freqBandIndicatorPriority-r12 | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| hyperSFN-r13 | Not present |  |  |
| eDRX-Allowed-r13 | Not present |  |  |
| eDRX-Allowed-r13 | True |  | eDRX |
| cellSelectionInfoCE-r13 | CellSelectionInfoCE-r13-DEFAULT |  |  |
| bandwidthReducedAccessRelatedInfo-r13 SEQUENCE { |  |  |  |
| si-WindowLength-BR-r13 | ms20 | To allow sufficient number of retransmissions. |  |
| si-RepetitionPattern-r13 | every2ndRF | ENUMERATED {everyRF, every2ndRF, every4thRF, every8thRF} |  |
| schedulingInfoList-BR-r13 SEQUENCE (SIZE (1..maxSI-Message)) OF SEQUENCE { | n entries | n is the same number of entries, and listed in the same order, as in SchedulingInfoList (without suffix) |  |
| si-Narrowband-r13[1] | 1 |  |  |
| si-Narrowband-r13[1] | 2 |  | 20MHz BW |
| si-TBS-r13[1] | b712 |  |  |
| …. |  |  |  |
| si-Narrowband-r13[n] | 1 |  |  |
| si-Narrowband-r13[n] | 2 |  | 20MHz BW |
| si-TBS-r13[n] | b712 |  |  |
| } |  |  |  |
| fdd-DownlinkOrTddSubframeBitmapBR-r13 | Not present | All non-MBSFN subframes are considered as valid subframes for FDD downlink or TDD transmissions |  |
| fdd-UplinkSubframeBitmapBR-r13 | Not present | All FDD uplink subframes are considered as valid subframes |  |
| startSymbolBR-r13 | 3 |  | FDD |
|  | 2 |  | TDD |
| si-HoppingConfigCommon-r13 | off |  |  |
| si-ValidityTime-r13 | True |  |  |
| systemInfoValueTagList-r13 SEQUENCE (SIZE (1..maxSI-Message)) OF | n entries | n is the same number of entries, and listed in the same order, as in SchedulingInfoList (without suffix) |  |
| SystemInfoValueTagSI-r13[1] | 0 |  |  |
| …. |  |  |  |
| SystemInfoValueTagSI-r13[n] | 0 |  |  |
| } |  |  |  |
| nonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  | FREQ HOPPING |
| freqHoppingParametersDL-r13 SEQUENCE { |  |  |  |
| mpdcch-pdsch-HoppingNB-r13 | nb4 | 4 narrowbands |  |
| interval-DLHoppingConfigCommonModeA-r13 CHOICE { |  |  |  |
| interval-FDD-r13 | int4 |  | FDD |
| interval-TDD-r13 | Int5 |  | TDD |
| } |  |  |  |
| interval-DLHoppingConfigCommonModeB-r13 CHOICE { |  |  |  |
| interval-FDD-r13 | int16 |  | FDD |
| interval-TDD-r13 | int20 |  | TDD |
| } |  |  |  |
| mpdcch-pdsch-HoppingOffset-r13 | 2 | INTEGER (1..16 (maxAvailNarrowBands-r13)) |  |
| } |  |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |
| QBASED | This condition applies to Quality based cell (re)selection signalling test cases. |
| UECAT0 | This condition applies when UE under test is a UE of Category 0. |
| eDRX | This condition applies when UE under test is in enhanced coverage and eDRX is allowed. |
| RF | For RF, performance and RRM testing |
| SIG | For Signalling tests |
| FREQ HOPPING | This condition applies when MPDCCH/PDSCH frequency hopping is used |
| NORMAL COVERAGE | Normal coverage testing |
| ENHANCED COVERAGE | This condition applies for enhanced coverage testing |
| 20MHz BW | Used for cells with 20MHz bandwidth |
| Band > 64 | If band > 64 is selected |

#### 4.4.3.3	Common contents of system information blocks

#### -	SystemInformationBlockType2

The IE SystemInformationBlockType2 contains radio resource configuration information that is common for all UEs.

Table 4.4.3.3-1: SystemInformationBlockType2

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| ac-BarringInfo SEQUENCE {} | Not present |  |  |
| radioResourceConfigCommon SEQUENCE {} | RadioResourceConfigCommonSIB-DEFAULT | See subclause 4.6.3 |  |
| ue-TimersAndConstants SEQUENCE { |  |  |  |
| t300 | ms1000 | Typical value in real network |  |
| t301 | ms1000 | Typical value in real network |  |
| t310 | ms1000 | Typical value in real network |  |
| n310 | n1 |  |  |
| t311 | ms10000 | Typical value in real network |  |
| n311 | n1 |  |  |
| } |  |  |  |
| freqInfo SEQUENCE { |  |  |  |
| ul-CarrierFreq | Not present | Default UL EARFCN applies |  |
| ul-Bandwidth | Not Present |  |  |
| additionalSpectrumEmission | 1 (NS_01) | A-MPR doesn’t apply by default. See TS 36.101 table 6.2.4-1. |  |
| additionalSpectrumEmission | 32 | When IE AdditionalSpectrumEmission - v10l0 is used, 32 should be set to IE AdditionalSpectrumEmission. | NS value > 32 |
| } |  |  |  |
| mbsfn-SubframeConfigList | Not present |  |  |
| mbsfn-SubframeConfigList SEQUENCE (SIZE (1..maxMBSFN-Allocations)) OF SEQUENCE { |  |  | MBMS |
| radioframeAllocationPeriod | n4 |  |  |
| radioframeAllocationOffset | 1 |  | FDD |
|  | 0 |  | TDD |
| subframeAllocation CHOICE{ |  |  |  |
| oneFrame | ‘100000’ B |  | FDD |
|  | ‘000010’ B |  | TDD |
| } |  |  |  |
| } |  |  |  |
| timeAlignmentTimerCommon | sf750 | ‘sf750’ is applicable to the widest range of mobility (up to about 360km/h). |  |
| lateNonCriticalExtension SEQUENCE { | Not present |  |  |
| lateNonCriticalExtension SEQUENCE { |  | OCTET STRING (CONTAINING SystemInformationBlockType2-v8h0-IEs) | NS value > 32 |
| multiBandInfoList | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ul-CarrierFreq-v9e0 | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  | OCTET STRING (CONTAINING SystemInformationBlockType2-v10m0-IEs) |  |
| freqInfo-v10l0 SEQUENCE { |  |  |  |
| additionalSpectrumEmission-v10l0 | Specify the network signaling value corresponding to the E-UTRA Band defined in TS 36.101 table 6.2.4-1 |  |  |
| multiBandInfoList-v10l0 | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| Dummy | Not present | This field is not used in the specification. If received it shall be ignored by the UE. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| unicastFreqHoppingInd-r13 | true | Enabled only for CE Mode test cases requiring frequency hopping. | CEfreqHopp |
| cIoT-EPS-OptimisationInfo-r13 SEQUENCE (SIZE (1.. maxPLMN-r11)) OF SEQUENCE { |  |  | CIoT-test |
| up-CIoT-EPS-Optimisation-r13 | true |  |  |
| cp-CIoT-EPS-Optimisation-r13 | true |  |  |
| attachWithoutPDN-Connectivity-r13 | false |  |  |
| } |  |  |  |
| useFullResumeID-r13 | Not present |  |  |
| plmn-InfoList-r15 | Not present |  |  |
| plmn-InfoList-r15 SEQUENCE (SIZE (1..maxPLMN-r11)) OF SEQUENCE { | 1 entry |  | 5G |
| upperLayerIndication-r15[1] | true |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| MBMS | MBMS cell environment |
| FDD | FDD cell environment |
| TDD | TDD cell environment |
| CEfreqHopp | Used when frequency hopping is used in CE test cases |
| CIoT-test | CIoT test environment |
| NS value > 32 | When A-MPR applies with Network Signaling value > 32 |
| 5G | Applicable to 5G test cases defined in TS 38.523-1 [71] |

#### -	SystemInformationBlockType3

The IE SystemInformationBlockType3 contains cell re-selection information common for intra-frequency, inter-frequency and/or inter-RAT cell re-selection (i.e. applicable for more than one type of cell re-selection but not necessarily all) as well as intra-frequency cell re-selection information other than neighbouring cell related.

Table 4.4.3.3-2: SystemInformationBlockType3

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |  |
| cellReselectionInfoCommon SEQUENCE { |  |  |  |  |
| q-Hyst | dB0 | To reduce interference between intra-frequency multiple cells |  |  |
| speedStateReselectionPars SEQUENCE {} | Not present |  |  |  |
| } |  |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |  |
| s-NonIntraSearch | Not present |  |  |  |
| threshServingLow | 0 (0 dB) | Typical value in real network |  |  |
| cellReselectionPriority | 4 | A middle value in the range has been selected. |  |  |
| } |  |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |  |
| q-RxLevMin | -70 (-140 dBm) | For RF/RRM test cases |  |  |
|  | -106 dBm | For signalling test cases |  |  |
| p-Max | Not present |  |  |  |
| s-IntraSearch | Not present |  |  |  |
| allowedMeasBandwidth | Not present | The downlink bandwidth of the serving cell applies. |  |  |
| presenceAntennaPort1 | FALSE |  |  |  |
|  | TRUE | At least two cell-specific antenna ports are used in all neighbouring cells. | All neighCells with port1 |  |
| neighCellConfig | '01'B (No MBSFN subframes are present in all neighbour cells) | MBSFN doesn’t apply by default. |  |  |
| t-ReselectionEUTRA | 0 | Typical value in real network |  |  |
| t-ReselectionEUTRA-SF | Not present |  |  |  |
| } |  |  |  |  |
| lateNonCriticalExtension { |  |  |  |
| s-IntraSearch-v920 SEQUENCE {} | Not present |  |  |
| s-NonIntraSearch-v920 SEQUENCE {} | Not present |  |  |
| q-QualMin-r9 | Not present |  |  |
|  | -20 (-20dB) |  | QBASED |
| threshServingLowQ-r9 | Not present |  |  |
|  | 3 (3dB) |  | QBASED |
| cellSelectionInfoCE-r13 | CellSelectionInfoCE-r13-DEFAULT |  | CEmodeA, CEmodeB |
|  | Not present |  |  |
| t-ReselectionEUTRA-CE-r13 | 0 |  | CEmodeA, CEmodeB |
|  | Not present |  |  |
| } |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| QBASED | See the definition below table 4.4.3.2-3. |
| All neighCells with port1 | Used for all neighbouring cells with at least two cell-specific antenna ports |
| CEmodeA | CE mode A test environment |
| CEmodeB | CE mode B test environment |

#### -	SystemInformationBlockType4

The IE SystemInformationBlockType4 contains neighbouring cell related information relevant only for intra-frequency cell re-selection. The IE includes cells with specific re-selection parameters as well as blacklisted cells.

Table 4.4.3.3-3: SystemInformationBlockType4

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType4 ::= SEQUENCE { |  |  |  |
| intraFreqNeighCellList SEQUENCE (SIZE (1..maxCellIntra)) OF SEQUENCE {} | Not present | Not required unless Qoffset configuration is tested. When Qoffset configuration is tested, see table 6.3.1.1-1. |  |
| intraFreqBlackCellList SEQUENCE (SIZE (1..maxCellBlack)) OF SEQUENCE {} | Not present | Not required unless Blacklisted cell list configuration is tested. When Blacklisted cell list configuration is tested, see table 6.3.1.1-1. |  |
| csg-PhysCellIdRange SEQUENCE {} | Not present |  | NonCSG |
| csg-PhysCellIdRange SEQUENCE { |  |  | CSG |
| start | Physical Cell ID of the cell on which this SIB is transmitted |  |  |
| range | Not present | The UE shall apply value 1 in case the field is absent, in which case only the physical cell identity value indicated by start applies. |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| NonCSG | SIB4 transmitted on a non-CSG cell |
| CSG | SIB4 transmitted on a CSG cell |

#### -	SystemInformationBlockType5

The IE SystemInformationBlockType5 contains information relevant only for inter-frequency cell re-selection i.e. information about other E-UTRA frequencies and inter-frequency neighbouring cells relevant for cell re-selection. The IE includes cell re-selection parameters common for a frequency as well as cell specific re-selection parameters.

Table 4.4.3.3-4: SystemInformationBlockType5

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | The same number of entries as the configured inter-freq carriers. For Signalling test cases except NAS, see table 6.3.1.2-1. For NAS test cases when cells are on same PLMN, see table 6.3.2.3.2-1. | n denotes the index of the entry |  |  |
| dl-CarrierFreq[n] | Downlink EARFCN under test For Signalling test cases except NAS, see table 6.3.1.2-1. For NAS test cases when cells are on same PLMN, see table 6.3.2.3.2-1. |  |  |  |
| dl-CarrierFreq[n] | 65535 | maxEARFCN | Band > 64 |  |
| q-RxLevMin[n] | -65 (-130 dBm) | For RF/RRM test cases |  |  |
|  | -106 dBm | For signalling test cases |  |  |
| p-Max[n] | Not present |  |  |  |
| t-ReselectionEUTRA[n] | 0 | Typical value in real network |  |  |
| t-ReselectionEUTRA-SF[n] | Not present | Not required unless speed-dependent cell re-selection is tested. |  |  |
| threshX-High[n] | 2 (4 dB) | This value should be higher than threshServingLow of the serving cell to avoid ping-pong with lower priority cells. |  |  |
| threshX-Low[n] | 1 (2 dB) |  |  |  |
| allowedMeasBandwidth[n] | See subclause 4.4.3.4 | Channel-bandwidth-dependent parameter |  |  |
| presenceAntennaPort1[n] | FALSE |  |  |  |
|  | TRUE | At least two cell-specific antenna ports are used in all neighbouring cells. | All neighCells with port1 |  |
| cellReselectionPriority[n] | 4 | The same priority as the one used for serving cell in SIB 3. |  |  |
| neighCellConfig[n] | '01'B (No MBSFN subframes are present in all neighbour cells) | MBSFN doesn’t apply by default. |  |  |
| q-OffsetFreq[n] | dB0 | Qoffset doesn’t apply by default. |  |  |
| interFreqNeighCellList[n] SEQUENCE (SIZE (1..maxCellInter)) OF SEQUENCE {} | Not present | Not required unless Qoffset configuration is tested. |  |  |
| interFreqBlackCellList[n] SEQUENCE (SIZE (1..maxCellBlack)) OF SEQUENCE {} | Not present | Not required unless Blacklisted cell list configuration is tested. |  |  |
| q-QualMin-r9[n] | Not present |  |  |
|  | -20 (-20dB) |  | QBASED |
| threshX-Q-r9[n] SEQUENCE {} | Not present |  |  |
| threshX-Q-r9[n] SEQUENCE { |  |  | QBASED |
| threshX-HighQ-r9 | 5 (5dB) |  |  |
| threshX-LowQ-r9 | 5 (5dB) |  |  |
| } |  |  |  |
| } |  |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | Band > 64 |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE { (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |  |
| dl-CarrierFreq-v9e0[n] | Downlink EARFCN under test For Signalling test cases except NAS, see table 6.3.1.2-1. For NAS test cases when cells are on same PLMN, see table 6.3.2.3.2-1 |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| interFreqCarrierFreqList-v1250[n] SEQUENCE {} | Not present |  |  |  |
| interFreqCarrierFreqListExt-r12[n] SEQUENCE {} | Not present |  |  |  |
| interFreqCarrierFreqListExt-v1280[n] SEQUENCE {} | Not present |  |  |  |
| interFreqCarrierFreqList-v1310[n] SEQUENCE {} | Not present |  |  |  |
| interFreqCarrierFreqListExt-v1310[n] SEQUENCE {} | Not present |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| QBASED | See the definition below table 4.4.3.2-3. |
| All neighCells with port1 | Used for all neighbouring cells with at least two cell-specific antenna ports |
| Band > 64 | If band > 64 is selected |

#### -	SystemInformationBlockType6

The IE SystemInformationBlockType6 contains information relevant only for inter-RAT cell re-selection i.e. information about UTRA frequencies and UTRA neighbouring cells relevant for cell re-selection. The IE includes cell re-selection parameters common for a frequency.

Table 4.4.3.3-5: SystemInformationBlockType6

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| SystemInformationBlockType6 ::= SEQUENCE { |  |  |  |  |
| carrierFreqListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE {} | Not present |  | UTRA-TDD |  |
| carrierFreqListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { | The same number of entries as the configured UTRA FDD carriers For Signalling test cases, see table 6.3.1.3-1 | n denotes the index of the entry | UTRA-FDD |  |
| carrierFreq[n] | Downlink UARFCN under test For Signalling test cases, see table 6.3.1.3-1 |  |  |  |
| cellReselectionPriority[n] | 3 |  |  |  |
| threshX-High[n] | 2 (4 dB) |  |  |  |
| threshX-Low[n] | 1 (2 dB) |  |  |  |
| q-RxLevMin[n] | -40 (-79 dBm) | The same value as defined in TS 34.108 [5], table 6.1.1. |  |  |
| p-MaxUTRA[n] | 21 (21 dBm) | The same value as defined in TS 34.108 [5], table 6.1.1. |  |  |
| q-QualMin[n] | -24 (-24 dB) | The same value as defined in TS 34.108 [5], table 6.1.1. |  |  |
| threshX-Q-r9[n] SEQUENCE {} | Not present |  |  |
| threshX-Q-r9[n] SEQUENCE { |  |  | QBASED |
| threshX-HighQ-r9 | 9 (9dB) |  |  |
| threshX-LowQ-r9 | 9 (9dB) |  |  |
| } |  |  |  |
| } |  |  |  |  |
| carrierFreqListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE {} | Not present |  | UTRA-FDD |  |
| carrierFreqListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { | The same number of entries as the configured UTRA TDD carriers For Signalling test cases, see table 6.3.1.3-1 | n denotes the index of the entry | UTRA-TDD |  |
| carrierFreq[n] | Downlink UARFCN under test For Signalling test cases, see table 6.3.1.3-1 |  |  |  |
| cellReselectionPriority[n] | 3 |  |  |  |
| threshX-High[n] | 2 (4 dB) |  |  |  |
| threshX-Low[n] | 1 (2 dB) |  |  |  |
| q-RxLevMin[n] | -41 (-81 dBm) | The same value as defined in TS 34.108 [5], table 6.1.6a |  |  |
| p-MaxUTRA[n] | 21 (21 dBm) | The same value as defined in TS 34.108 [5], table 6.1.6a |  |  |
| } |  |  |  |  |
| t-ReselectionUTRA | 0 | Typical value in real network |  |  |
| t-ReselectionUTRA-SF | Not present |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |
| QBASED | See the definition below table 4.4.3.2-3. |

#### -	SystemInformationBlockType7

The IE SystemInformationBlockType7 contains information relevant only for inter-RAT cell re-selection i.e. information about GERAN frequencies relevant for cell re-selection. The IE includes cell re-selection parameters for each frequency.

Table 4.4.3.3-6: SystemInformationBlockType7

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType7 ::= SEQUENCE { |  |  |  |
| t-ReselectionGERAN | 0 |  |  |
| t-ReselectionGERAN-SF | Not present |  |  |
| carrierFreqsInfoListSEQUENCE (SIZE (1..maxGNFG)) OF SEQUENCE { | The same number of entries as the configured GERAN carriers For RRM test cases, see table 7.2.2.1-1 For Signalling test cases, see table 6.3.1.4-1 | n denotes the index of the entry |  |
| carrierFreqs [n] SEQUENCE { |  |  |  |
| startingARFCN[n] | Set the corresponding starting ARFCN of the GERAN cells under test. For RRM test cases, see table 7.2.2.1-1 For Signalling test cases, see table 6.3.1.4-1 |  |  |
| bandIndicator[n] | Set according to the band used for GERAN cells under test |  |  |
| followingARFCNs[n] CHOICE { |  |  |  |
| explicitListOfARFCNs[n] | Set the corresponding ARFCN of GERAN cells under test For RRM test cases, see table 7.2.2.1-1 For Signalling test cases, see table 6.3.1.4-1 |  |  |
| } |  |  |  |
| } |  |  |  |
| commonInfo[n] SEQUENCE { |  |  |  |
| cellReselectionPriority[n] | 2 |  |  |
| ncc-Permitted[n] | ‘01000000'B | NCC=1 permitted |  |
| q-RxLevMin[n] | 2 |  |  |
| p-MaxGERAN[n] | 0 |  |  |
| threshX-High[n] | 2 |  |  |
| threshX-Low[n] | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### -	SystemInformationBlockType8

The IE SystemInformationBlockType8 contains information relevant only for inter-RAT cell re-selection i.e. information about CDMA2000 frequencies and CDMA2000 neighbouring cells relevant for cell re-selection. The IE includes cell re-selection parameters common for a frequency as well as cell specific re-selection parameters.

Table 4.4.3.3-7: SystemInformationBlockType8

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType8 ::= SEQUENCE { |  |  |  |
| systemTimeInfo SEQUENCE { |  |  |  |
| cdma-EUTRA-Synchronisation | TRUE |  |  |
| cdma-SystemTime CHOICE { |  |  |  |
| synchronousSystemTime | A valid value as per TS 36.331 and calculated by the SS |  |  |
| } |  |  |  |
| } |  |  |  |
| searchWindowSize | 5 |  |  |
| parametersHRPD SEQUENCE {} | Not present |  | 1XRTT |
| parametersHRPD SEQUENCE { |  |  | HRPD |
| preRegistrationInfoHRPD SEQUENCE { |  |  |  |
| preRegistrationAllowed | FALSE |  |  |
| preRegistrationZoneId | Not present |  |  |
| secondaryPreRegistrationZoneIdList | Not present |  |  |
| } |  |  |  |
| cellReselectionParametersHRPD SEQUENCE { |  |  |  |
| bandClassList SEQUENCE (SIZE (1..maxCDMA -BandClass)) OF SEQUENCE { | 1 entry |  |  |
| bandClass | Operating band class under test. | ENUMERATED {bc0, bc1, bc2, bc3, bc4, bc5, bc6, bc7, bc8, bc9, bc10, bc11, bc12, bc13, bc14, bc15, bc16, bc17, spare14, spare13, spare12, spare11, spare10, spare9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1, ...} |  |
| cellReselectionPriority | 1 |  |  |
| threshX-High | 30(-30) | INTEGER (0..63) |  |
| threshX-Low | 32(-32) | INTEGER (0..63) |  |
| } |  |  |  |
| neighCellList SEQUENCE (SIZE (1..16)) OF SEQUENCE { | 1 entry |  |  |
| bandClass | Operating band class under test | ENUMERATED {bc0, bc1, bc2, bc3, bc4, bc5, bc6, bc7, bc8, bc9, bc10, bc11, bc12, bc13, bc14, bc15, bc16, bc17, spare14, spare13, spare12, spare11, spare10, spare9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1, ...} |  |
| neighCellsPerFreqList SEQUENCE (SIZE (1..16)) OF SEQUENCE { | The same number of entries as the configured CDMA2000 HRPD frequencies For Signalling test cases, see table 6.3.1.5-1 | n denotes the index of the entry |  |
| arfcn[n] | ARFCN of the CDMA2000 HRPD frequency For Signalling test cases, see table 6.3.1.5-1 | INTEGER (0..2047) |  |
| physCellIdList[n] SEQUENCE (SIZE (1..16)) OF {INTEGER (0..maxPNOffset) } | Physical Cell ID List of the CDMA2000 HRPD frequency For Signalling test cases, see table 6.3.1.5-1 | INTEGER (0..maxPNOffset) |  |
| } |  |  |  |
| } |  |  |  |
| t-ReselectionCDMA2000 | 0 | INTEGER (0..7) |  |
| t-ReselectionCDMA2000-SF | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| parameters1XRTT SEQUENCE {} | Not present |  | HRPD |
| parameters1XRTT SEQUENCE { |  |  | 1XRTT |
| csfb-RegistrationParam1XRTT SEQUENCE { |  |  |  |
| longCodeState1XRTT | A valid value, calculated and updated by the SS | BIT STRING (SIZE (42)) OPTIONAL |  |
| cellReselectionParameters1XRTT SEQUENCE { |  |  |  |
| bandClassList SEQUENCE (SIZE (1..maxCDMA -BandClass)) OF SEQUENCE { | 1 entry |  |  |
| bandClass | Operating band class under test | ENUMERATED {bc0, bc1, bc2, bc3, bc4, bc5, bc6, bc7, bc8, bc9, bc10, bc11, bc12, bc13, bc14, bc15, bc16, bc17, spare14, spare13, spare12, spare11, spare10, spare9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1, ...} |  |
| cellReselectionPriority | 0 |  |  |
| threshX-High | 30(-30) | INTEGER (0..63) |  |
| threshX-Low | 32(-32) | INTEGER (0..63) |  |
| } |  |  |  |
| neighCellList SEQUENCE (SIZE (1..16)) OF SEQUENCE { | 1 entry |  |  |
| bandClass | Operating band class under test | ENUMERATED {bc0, bc1, bc2, bc3, bc4, bc5, bc6, bc7, bc8, bc9, bc10, bc11, bc12, bc13, bc14, bc15, bc16, bc17, spare14, spare13, spare12, spare11, spare10, spare9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1, ...} |  |
| neighCellsPerFreqList SEQUENCE (SIZE (1..16)) OF SEQUENCE { | The same number of entries as the configured CDMA2000 1xRTT frequencies For Signalling test cases, see table 6.3.1.6-1 | n denotes the index of the entry |  |
| arfcn[n] | ARFCN of the CDMA2000 1xRTT frequency For Signalling test cases, see table 6.3.1.6-1 | INTEGER (0..2047) |  |
| physCellIdList[n] SEQUENCE (SIZE (1..16)) OF {INTEGER (0..maxPNOffset) } | Physical Cell ID List of the CDMA2000 1xRTT frequency For Signalling test cases, see table 6.3.1.6-1 | INTEGER (0..maxPNOffset) |  |
| } |  |  |  |
| } |  |  |  |
| t-ReselectionCDMA2000 | 0 | INTEGER (0..7) |  |
| t-ReselectionCDMA2000-SF | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| HRPD | CDMA2000 HRPD cell environment |
| 1XRTT | CDMA2000 1XRTT cell environment |

#### -	SystemInformationBlockType9

The IE SystemInformationBlockType9 contains a home eNB name (HNB Name).

Table 4.4.3.3-8: SystemInformationBlockType9

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType9 ::= SEQUENCE { |  |  |  |
| hnb-Name | “3gppTest” |  |  |
| } |  |  |  |

#### -	SystemInformationBlockType10

The IE SystemInformationBlockType10 contains an ETWS primary notification.

Table 4.4.3.3-9: SystemInformationBlockType10

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType10 ::= SEQUENCE { |  |  |  |
| messageIdentifier | '0001 0001 0000 0010'B | ETWS message identifier for earthquake and tsunami message [see TS 23.041] |  |
| serialNumber - Geographical Scope (2 bit) - Message Code (10 bit) - Emergency User Alert (1 bit) - Popup (1 bit) - Update Number (4 bit) | '0011 0000 0000 0000'B Octet 1 bit 7 ~ 6 Octet 1 bit 5 ~ Octet 2 bit 4 Octet 1 bit 5 Octet 1 bit 4 Octet 2 bit 3 ~ 0 | Cell wide; '11 0000 0000'B Activate emergency user alert; Activate popup; For each update, incremented by one. [see TS 23.041] |  |
| warningType - Warning Type Value (7 bit) - Emergency User Alert (1 bit) - Popup (1 bit) - Padding (7 bit) | '0000 0101 1000 0000'B Octet 1 bit 7 ~ 1 Octet 1 bit 0 Octet 2 bit 7 Octet 2 bit 6 ~ bit 0 | Earthquake and Tsunami; Activate emergency user alert; Activate Popup. [see TS 23.041] '000 0000'B |  |
| warningSecurityInfo | Not present |  |  |
| } |  |  |  |

#### -	SystemInformationBlockType11

The IE SystemInformationBlockType11 contains an ETWS secondary notification.

Table 4.4.3.3-10: SystemInformationBlockType11 (1st Segment)

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType11 ::= SEQUENCE { |  |  |  |
| messageIdentifier | '0001 0001 0000 0010'B | ETWS message identifier for earthquake and tsunami message [see TS 23.041] |  |
| serialNumber - Geographical Scope (2 bit) - Message Code (10 bit) - Emergency User Alert (1 bit) - Popup (1 bit) - Update Number (4 bit) | '0011 0000 0000 0000'B Octet 1 bit 7 ~ 6 Octet 1 bit 5 ~ Octet 2 bit 4 Octet 1 bit 5 Octet 1 bit 4 Octet 2 bit 3 ~ 0 | Cell wide; '11 0000 0000'B Activate emergency user alert; Activate popup; For each update, incremented by one. [see TS 23.041] |  |
| warningMessageSegmentType | notLastSegment |  |  |
| warningMessageSegmentNumber | 0 |  |  |
| warningMessageSegment | Octetstring of N | where N ≥ 1 and less than 1246. [see TS 23.041] |  |
| dataCodingScheme | Bitstring (8) ID of the alphabet/coding and the applied language [see TS 23.041] | Provided as PIXITs [see TS 36.523-3 [20] cl. 9] |  |
| } |  |  |  |

Table 4.4.3.3-11: SystemInformationBlockType11 (2nd Segment)

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType11 ::= SEQUENCE { |  |  |  |
| messageIdentifier | '0001 0001 0000 0010'B | ETWS message identifier for earthquake and tsunami message [see TS 23.041] |  |
| serialNumber - Geographical Scope (2 bit) - Message Code (10 bit) - Emergency User Alert (1 bit) - Popup (1 bit) - Update Number (4 bit) | '0011 0000 0000 0000'B Octet 1 bit 7 ~ 6 Octet 1 bit 5 ~ Octet 2 bit 4 Octet 1 bit 5 Octet 1 bit 4 Octet 2 bit 3 ~ 0 | Cell wide; '11 0000 0000'B Activate emergency user alert; Activate popup; For each update, incremented by one. [see TS 23.041] |  |
| warningMessageSegmentType | notLastSegment |  |  |
| warningMessageSegmentNumber | 1 |  |  |
| warningMessageSegment | Octetstring of N | where N ≥ 1 and less than 1246. [see TS 23.041] |  |
| dataCodingScheme | Not present |  |  |
| } |  |  |  |

Table 4.4.3.3-12: SystemInformationBlockType11 (3rd Segment)

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType11 ::= SEQUENCE { |  |  |  |
| messageIdentifier | '0001 0001 0000 0010'B | ETWS message identifier for earthquake and tsunami message [see TS 23.041] |  |
| serialNumber - Geographical Scope (2 bit) - Message Code (10 bit) - Emergency User Alert (1 bit) - Popup (1 bit) - Update Number (4 bit) | '0011 0000 0000 0000'B Octet 1 bit 7 ~ 6 Octet 1 bit 5 ~ Octet 2 bit 4 Octet 1 bit 5 Octet 1 bit 4 Octet 2 bit 3 ~ 0 | Cell wide; '11 0000 0000'B Activate emergency user alert; Activate popup; For each update, incremented by one. [see TS 23.041] |  |
| warningMessageSegmentType | lastSegment |  |  |
| warningMessageSegmentNumber | 2 |  |  |
| warningMessageSegment | Octetstring of N | where N ≥ 1 and less than 1246. [see TS 23.041] |  |
| dataCodingScheme | Not present |  |  |
| } |  |  |  |

#### -	SystemInformationBlockType12

The IE SystemInformationBlockType12contains an CMAS  notification.

Table 4.4.3.3-12a: SystemInformationBlockType12 (1st Segment)

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType11 ::= SEQUENCE { |  |  |  |
| messageIdentifier-r9 | '0001 0001 0001 0010'B | CMAS CBS Message Identifier for CMAS Presidential Level Alerts (see TS 23.041]) |  |
| serialNumber-r9 | '0011 0000 0000 0000'B |  |  |
| warningMessageSegmentType | notLastSegment |  |  |
| warningMessageSegmentNumber | 0 |  |  |
| warningMessageSegment | Octetstring of N | where N ≥ 1 and less than 1246. (see TS 23.041) |  |
| dataCodingScheme | -Bitstring (8) ID of the alphabet/coding and the applied language [see TS 23.041] | Provided as PIXITs [see TS 36.523-3 [20] cl. 9] |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

Table 4.4.3.3-12b: SystemInformationBlockType12 (2nd Segment)

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType11 ::= SEQUENCE { |  |  |  |
| messageIdentifier-r9 | '0001 0001 0001 0010'B | CMAS CBS Message Identifier for CMAS Presidential Level Alerts (see TS 23.041) |  |
| serialNumber-r9 | '0011 0000 0000 0000'B |  |  |
| warningMessageSegmentType | notLastSegment |  |  |
| warningMessageSegmentNumber | 1 |  |  |
| warningMessageSegment | Octetstring of N | where N ≥ 1 and less than 1246. (see TS 23.041) |  |
| dataCodingScheme | Not present |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

Table 4.4.3.3-12c: SystemInformationBlockType12 (3rd Segment)

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType11 ::= SEQUENCE { |  |  |  |
| messageIdentifier-r9 | '0001 0001 0001 0010'B | CMAS CBS Message Identifier for CMAS Presidential Level Alerts (see TS 23.041) |  |
| serialNumber-r9 | '0011 0000 0000 0000'B |  |  |
| warningMessageSegmentType | lastSegment |  |  |
| warningMessageSegmentNumber | 2 |  |  |
| warningMessageSegment | Octetstring of N | where N ≥ 1 and less than 1246. (see TS 23.041) |  |
| dataCodingScheme | Not present |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

#### -	SystemInformationBlockType13

The IE SystemInformationBlockType13 contains the information required to acquire the MBMS control information associated with one or more MBSFN areas.

Table 4.4.3.3-13: SystemInformationBlockType13

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType13 ::= SEQUENCE { |  |  |  |
| mbsfn-AreaInfoList-r9 SEQUENCE (SIZE(1..maxMBSFN-Area)) OF SEQUENCE { |  |  |  |
| mbsfn-AreaId-r9 | 0 |  |  |
| non-MBSFNregionLength | 2 |  |  |
| notificationIndicator-r9 | 0 |  |  |
| mcch-Config-r9 SEQUENCE { |  |  |  |
| mcch-RepetitionPeriod-r9 | rf32 |  |  |
| mcch-Offset-r9 | 1 |  | FDD |
|  | 0 |  | TDD |
| mcch-ModificationPeriod-r9 | rf512 |  |  |
| sf-AllocInfo-r9 | ‘100000’B |  | FDD |
|  | ‘000010’B |  | TDD |
| signallingMCS-r9 | n2 |  |  |
| } |  |  |  |
| } |  |  |  |
| notificationConfig-r9 SEQUENCE { |  |  |  |
| notificationRepetitionCoeff-r9 | n4 |  |  |
| notificationOffset-r9 | 1 |  | FDD |
|  | 0 |  | TDD |
| notificationSF-Index-r9 | 1 | Subframe #1 | FDD |
|  | 5 | Subframe #9 | TDD |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |

#### -	SystemInformationBlockType14

The IE SystemInformationBlockType14 contains the EAB parameters.

Table 4.4.3.3-13A: SystemInformationBlockType14

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType14-r11 ::= SEQUENCE { |  |  |  |
| eab-Param-r11 CHOICE { |  |  |  |
| EAB-Config-r11 ::= SEQUENCE { |  |  |  |
| eab-Category-r11 | a | EAB applies to all UEs in the PLMN ENUMERATED {a, b, c} |  |
| eab-BarringBitmap-r11 | '1111111111'B | All UE classes 0-9 BIT STRING (SIZE (10)) |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not Present |  |  |
| } |  |  |  |

#### -	SystemInformationBlockType15

The IE SystemInformationBlockType15 contains the MBMS Service Area Identities (SAI) of the current and/ or neighbouring carrier frequencies.

Table 4.4.3.3-14: SystemInformationBlockType15

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType15 ::= SEQUENCE { |  |  |  |
| mbms-SAI-IntraFreq-r11[1] SEQUENCE (SIZE (1..maxSAI-MBMS-r11)) OF { INTEGER (0..65535) } | 1 | 1 entry INTEGER (0..65535) |  |
| mbms-SAI-InterFreqList-r11 | Not present |  | MBMS_intraFreq, SCPTM_intraFreq |
| mbms-SAI-InterFreqList-r11[n] SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | The same number of entries as the configured inter-freq carriers providing the MBMS service in the test case (including additional bands, if any) | n denotes the index of the entry | MBMS_interFreq, SCPTM_interFreq |
| dl-CarrierFreq-r11 | Downlink EARFCN under test, see table 6.3.1.2-1. |  |  |
| mbms-SAI-List-r11[n] SEQUENCE (SIZE (1..maxSAI-MBMS-r11)) OF { INTEGER (0..65535) } | The same number of entries as the configured MBMS SAIs providing the MBMS service in the test case for the specific frequency | n denotes the index of the entry INTEGER (0..65535) |  |
| lateNonCriticalExtension | Not present |  | MBMS_intraFreq, MBMS_interFreq, SCPTM_intraFreq, SCPTM_interFreq |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| MBMS_intraFreq | SIB15 transmitted for a MBMS intra-band cell environment (MBMS service only provided for intra-frequency cells). |
| MBMS_interFreq | SIB15 transmitted for a MBMS inter-frequency cell environment (MBMS service provided on multiple frequencies within a band). |
| SCPTM_intraFreq | SIB15 transmitted for a SC-PTM intra-band cell environment (SC-PTM service only provided for intra-frequency cells). |
| SCPTM_interFreq | SIB15 transmitted for a SC-PTM inter-frequency cell environment (SC-PTM service provided on multiple frequencies within a band). |

#### -	SystemInformationBlockType17

The IE SystemInformationBlockType17 contains information relevant for RAN Assisted WLAN interworking

Table 4.4.3.3-15: SystemInformationBlockType17

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType17-r12 ::= SEQUENCE { |  |  |  |
| wlan-OffloadInfoPerPLMN-List-r12 [1] SEQUENCE (SIZE (1..maxPLMN-r11)) OF { |  | 1 Entry |  |
| wlan-OffloadConfigCommon-r12 | WLAN-OffloadConfig-DEFAULT |  |  |
| wlan-Id-List-r12 [1] SEQUENCE (SIZE (1..maxWLAN-Id-r12)) OF { |  | 1 Entry |  |
| wlan-Identifiers SEQUENCE{ |  |  |  |
| ssid-r12 | Set as per Table 4.4.8-1 | OCTET STRING (SIZE (1..32)) Service Set Identifier (SSID) defined in IEEE 802.11-2012 |  |
| bssid-r12 | Not present |  |  |
| hessid-r12 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

#### -	SystemInformationBlockType18

The IE SystemInformationBlockType18 contains the resource pool information for sidelink direct communication.

Table 4.4.3.3-16: SystemInformationBlockType18

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType18-r12 ::= SEQUENCE { |  |  |  |
| commConfig-r12 SEQUENCE { |  |  |  |
| commRxPool-r12 SEQUENCE (SIZE (1..maxSL-RxPool-r12)) OF SL-CommResourcePool-r12 { |  | 3 Rx pools |  |
| SL-CommResourcePool-r12[1] SEQUENCE { |  | RxPool 1 A monitoring UE can receive on the resources of this pool when a transmitting UE uses the TxPoolNormal 1 |  |
| sc-CP-Len-r12 | normal |  |  |
| sc-Period-r12 | sf40 |  |  |
| sc-TF-ResourceConfig-r12 SEQUENCE { |  |  |  |
| prb-Num-r12 | 13 |  | BW5 |
|  | 25 |  | BW10 |
| prb-Start-r12 | 0 |  |  |
| prb-End-r12 | 24 |  | BW5 |
|  | 49 |  | BW10 |
| offsetIndicator-r12 | 0 | small-r12 |  |
| subframeBitmap-r12 | 11000000 00000000 00000000 00000000 00000000 | bs40-r12 | FDD |
| } |  |  |  |
| data-CP-Len-r12 | normal |  |  |
| dataHoppingConfig-r12 SEQUENCE { |  |  |  |
| hoppingParameter-r12 | 0 |  |  |
| numSubbands-r12 | ns1 |  |  |
| rb-Offset-r12 | 0 |  |  |
| } |  |  |  |
| ue-SelectedResourceConfig-r12 SEQUENCE { |  |  |  |
| data-TF-ResourceConfig-r12 SEQUENCE { |  |  |  |
| prb-Num-r12 | 13 |  | BW5 |
|  | 25 |  | BW10 |
| prb-Start-r12 | 0 |  |  |
| prb-End-r12 | 24 |  | BW5 |
|  | 49 |  | BW10 |
| offsetIndicator-r12 | 0 | small-r12 |  |
| subframeBitmap-r12 | 00000000 00000000 11110000 00000000 00000000 | bs40-r12 | FDD |
| } |  |  |  |
| trpt-Subset-r12 | 001 |  |  |
| } |  |  |  |
| rxParametersNCell-r12 | Not present |  |  |
| txParameters-r12 | Not present |  |  |
| } |  |  |  |
| SL-CommResourcePool-r12[2] SEQUENCE { |  | RxPool 2 A monitoring UE can receive on the resources of this pool when a transmitting UE uses the TxPoolNormal 2 |  |
| sc-CP-Len-r12 | normal |  |  |
| sc-Period-r12 | sf40 |  |  |
| sc-TF-ResourceConfig-r12 SEQUENCE { |  |  |  |
| prb-Num-r12 | 13 |  | BW5 |
|  | 25 |  | BW10 |
| prb-Start-r12 | 0 |  |  |
| prb-End-r12 | 24 |  | BW5 |
|  | 49 |  | BW10 |
| offsetIndicator-r12 | 0 | small-r12 |  |
| subframeBitmap-r12 | 00110000 00000000 00000000 00000000 00000000 | bs40-r12 | FDD |
| } |  |  |  |
| data-CP-Len-r12 | normal |  |  |
| dataHoppingConfig-r12 SEQUENCE { |  |  |  |
| hoppingParameter-r12 | 0 |  |  |
| numSubbands-r12 | ns1 |  |  |
| rb-Offset-r12 | 0 |  |  |
| } |  |  |  |
| ue-SelectedResourceConfig-r12 SEQUENCE { |  |  |  |
| data-TF-ResourceConfig-r12 SEQUENCE { |  |  |  |
| prb-Num-r12 | 13 |  | BW5 |
|  | 25 |  | BW10 |
| prb-Start-r12 | 0 |  |  |
| prb-End-r12 | 24 |  | BW5 |
|  | 49 |  | BW10 |
| offsetIndicator-r12 | 0 | small-r12 |  |
| subframeBitmap-r12 | 00000000 00000000 00111100 00000000 00000000 | bs40-r12 | FDD |
| } |  |  |  |
| trpt-Subset-r12 | 001 |  |  |
| } |  |  |  |
| rxParametersNCell-r12 SEQUENCE { |  |  |  |
| tdd-Config-r12 SEQUENCE {} | Not present |  |  |
| syncConfigIndex-r12 | 1 | Entry 2 in discSyncConfig-r12 (SL-SyncConfig-r12[2]) |  |
| } |  |  |  |
| txParameters-r12 | Not present |  |  |
| } |  |  |  |
| SL-CommResourcePool-r12[3] SEQUENCE { |  | RxPool 3 A monitoring UE can receive on the resources of this pool when a transmitting UE uses the TxPoolExceptional 1 |  |
| sc-CP-Len-r12 | normal |  |  |
| sc-Period-r12 | sf40 |  |  |
| sc-TF-ResourceConfig-r12 SEQUENCE { |  |  |  |
| prb-Num-r12 | 13 |  | BW5 |
|  | 25 |  | BW10 |
| prb-Start-r12 | 0 |  |  |
| prb-End-r12 | 24 |  | BW5 |
|  | 49 |  | BW10 |
| offsetIndicator-r12 | 0 | small-r12 |  |
| subframeBitmap-r12 | 00001100 00000000 00000000 00000000 00000000 | bs40-r12 | FDD |
| } |  |  |  |
| data-CP-Len-r12 | normal |  |  |
| dataHoppingConfig-r12 SEQUENCE { |  |  |  |
| hoppingParameter-r12 | 0 |  |  |
| numSubbands-r12 | ns1 |  |  |
| rb-Offset-r12 | 0 |  |  |
| } |  |  |  |
| ue-SelectedResourceConfig-r12 SEQUENCE { |  |  |  |
| data-TF-ResourceConfig-r12 SEQUENCE { |  |  |  |
| prb-Num-r12 | 13 |  | BW5 |
|  | 25 |  | BW10 |
| prb-Start-r12 | 0 |  |  |
| prb-End-r12 | 24 |  | BW5 |
|  | 49 |  | BW10 |
| offsetIndicator-r12 | 0 | small-r12 |  |
| subframeBitmap-r12 | 00000000 00000000 00001111 00000000 00000000 | bs40-r12 | FDD |
| } |  |  |  |
| trpt-Subset-r12 | 001 |  |  |
| } |  |  |  |
| rxParametersNCell-r12 | Not present |  |  |
| txParameters-r12 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| commTxPoolNormalCommon-r12 SEQUENCE (SIZE (1..maxSL-TxPool-r12)) OF SL-CommResourcePool-r12 { |  | 2 Tx pools Normal |  |
| SL-CommResourcePool-r12[1] SEQUENCE { |  | TxPool 1 |  |
| sc-CP-Len-r12 | normal |  |  |
| sc-Period-r12 | sf40 |  |  |
| sc-TF-ResourceConfig-r12 SEQUENCE { |  |  |  |
| prb-Num-r12 | 13 |  | BW5 |
|  | 25 |  | BW10 |
| prb-Start-r12 | 0 |  |  |
| prb-End-r12 | 24 |  | BW5 |
|  | 49 |  | BW10 |
| offsetIndicator-r12 | 0 | small-r12 |  |
| subframeBitmap-r12 | 11000000 00000000 00000000 00000000 00000000 | bs40-r12 | FDD |
| } |  |  |  |
| data-CP-Len-r12 | normal |  |  |
| dataHoppingConfig-r12 SEQUENCE { |  |  |  |
| hoppingParameter-r12 | 0 |  |  |
| numSubbands-r12 | ns1 |  |  |
| rb-Offset-r12 | 0 |  |  |
| } |  |  |  |
| ue-SelectedResourceConfig-r12 SEQUENCE { |  |  |  |
| data-TF-ResourceConfig-r12 SEQUENCE { |  |  |  |
| prb-Num-r12 | 13 |  | BW5 |
|  | 25 |  | BW10 |
| prb-Start-r12 | 0 |  |  |
| prb-End-r12 | 24 |  | BW5 |
|  | 49 |  | BW10 |
| offsetIndicator-r12 | 0 | small-r12 |  |
| subframeBitmap-r12 | 00000000 00000000 11110000 00000000 00000000 | bs40-r12 | FDD |
| } |  |  |  |
| trpt-Subset-r12 | 001 |  |  |
| } |  |  |  |
| rxParametersNCell-r12 | Not present |  |  |
| txParameters-r12 SEQUENCE { |  |  |  |
| sc-TxParameters-r12 SEQUENCE { |  |  |  |
| alpha-r12 | al0 |  |  |
| p0-r12 | 31 |  |  |
| } |  |  |  |
| dataTxParameters-r12 SEQUENCE { |  |  |  |
| alpha-r12 | al0 |  |  |
| p0-r12 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| SL-CommResourcePool-r12[2] SEQUENCE { |  | TxPool 2 |  |
| sc-CP-Len-r12 | normal |  |  |
| sc-Period-r12 | sf40 |  |  |
| sc-TF-ResourceConfig-r12 SEQUENCE { |  |  |  |
| prb-Num-r12 | 13 |  | BW5 |
|  | 25 |  | BW10 |
| prb-Start-r12 | 0 |  |  |
| prb-End-r12 | 24 |  | BW5 |
|  | 49 |  | BW10 |
| offsetIndicator-r12 | 0 | small-r12 |  |
| subframeBitmap-r12 | 00110000 00000000 00000000 00000000 00000000 | bs40-r12 | FDD |
| } |  |  |  |
| data-CP-Len-r12 | normal |  |  |
| dataHoppingConfig-r12 SEQUENCE { |  |  |  |
| hoppingParameter-r12 | 0 |  |  |
| numSubbands-r12 | ns1 |  |  |
| rb-Offset-r12 | 0 |  |  |
| } |  |  |  |
| ue-SelectedResourceConfig-r12 SEQUENCE { |  |  |  |
| data-TF-ResourceConfig-r12 SEQUENCE { |  |  |  |
| prb-Num-r12 | 13 |  | BW5 |
|  | 25 |  | BW10 |
| prb-Start-r12 | 0 |  |  |
| prb-End-r12 | 24 |  | BW5 |
|  | 49 |  | BW10 |
| offsetIndicator-r12 | 0 | small-r12 |  |
| subframeBitmap-r12 | 00000000 00000000 00111100 00000000 00000000 | bs40-r12 | FDD |
| } |  |  |  |
| trpt-Subset-r12 | 001 |  |  |
| } |  |  |  |
| rxParametersNCell-r12 | Not present |  |  |
| txParameters-r12 SEQUENCE { |  |  |  |
| sc-TxParameters-r12 SEQUENCE { |  |  |  |
| alpha-r12 | al0 |  |  |
| p0-r12 | 31 |  |  |
| } |  |  |  |
| dataTxParameters-r12 SEQUENCE { |  |  |  |
| alpha-r12 | al0 |  |  |
| p0-r12 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| commTxPoolExceptional-r12 SEQUENCE (SIZE (1..maxSL-TxPool-r12)) OF SL-CommResourcePool-r12 { |  | 1 Tx pool Exceptional |  |
| SL-CommResourcePool-r12[1] SEQUENCE { |  | TxExceptionalPool 1 |  |
| sc-CP-Len-r12 | normal |  |  |
| sc-Period-r12 | sf40 |  |  |
| sc-TF-ResourceConfig-r12 SEQUENCE { |  |  |  |
| prb-Num-r12 | 13 |  | BW5 |
|  | 25 |  | BW10 |
| prb-Start-r12 | 0 |  |  |
| prb-End-r12 | 24 |  | BW5 |
|  | 49 |  | BW10 |
| offsetIndicator-r12 | 0 | small-r12 |  |
| subframeBitmap-r12 | 00001100 00000000 00000000 00000000 00000000 | bs40-r12 | FDD |
| } |  |  |  |
| data-CP-Len-r12 | normal |  |  |
| dataHoppingConfig-r12 SEQUENCE { |  |  |  |
| hoppingParameter-r12 | 0 |  |  |
| numSubbands-r12 | ns1 |  |  |
| rb-Offset-r12 | 0 |  |  |
| } |  |  |  |
| ue-SelectedResourceConfig-r12 SEQUENCE { |  |  |  |
| data-TF-ResourceConfig-r12 SEQUENCE { |  |  |  |
| prb-Num-r12 | 13 |  | BW5 |
|  | 25 |  | BW10 |
| prb-Start-r12 | 0 |  |  |
| prb-End-r12 | 24 |  | BW5 |
|  | 49 |  | BW10 |
| offsetIndicator-r12 | 0 | small-r12 |  |
| subframeBitmap-r12 | 00000000 00000000 00001111 00000000 00000000 | bs40-r12 | FDD |
| } |  |  |  |
| trpt-Subset-r12 | 001 |  |  |
| } |  |  |  |
| rxParametersNCell-r12 | Not present |  |  |
| txParameters-r12 SEQUENCE { |  |  |  |
| sc-TxParameters-r12 SEQUENCE { |  |  |  |
| alpha-r12 | al0 |  |  |
| p0-r12 | 31 |  |  |
| } |  |  |  |
| dataTxParameters-r12 SEQUENCE { |  |  |  |
| alpha-r12 | al0 |  |  |
| p0-r12 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| commSyncConfig-r12 SEQUENCE (SIZE (1..maxSL-SyncConfig-r12)) OF SL-SyncConfig-r12 { |  | 2 configuration reception/transmission of synchronisation signals |  |
| SL-SyncConfig-r12[1] SEQUENCE { |  | SyncConfig 1 To be used for transmission |  |
| syncCP-Len-r12 | normal |  |  |
| syncOffsetIndicator-r12 | 2 |  |  |
| slssid-r12 | 30 |  |  |
| txParameters-r12 SEQUENCE { |  |  |  |
| syncTxParameters-r12 SEQUENCE { |  |  |  |
| alpha-r12 | al0 |  |  |
| p0-r12 | 31 |  |  |
| } |  |  |  |
| syncTxThreshIC-r12 | 7 | -85dBm The threshold for starting transmission of SLSS |  |
| } |  |  |  |
| rxParamsNCell-r12 | Not present |  |  |
| } |  |  |  |
| SL-SyncConfig-r12[2] SEQUENCE { |  | SyncConfig 2 To be used for reception. RxPool Pool 2 is refering to this SyncConfig 2 |  |
| syncCP-Len-r12 | normal |  |  |
| syncOffsetIndicator-r12 | 25 |  |  |
| slssid-r12 | 40 |  |  |
| txParameters-r12 | Not Present |  |  |
| rxParamsNCell-r12 SEQUENCE { |  |  |  |
| physCellId-r12 | Set to the physical cell identity of the PCell |  |  |
| discSyncWindow-r12 | w1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |
| Note 1: For convenience the difference between the different pools is only in the relevant setting of the subframeBitmap. Note 2: Note that the resources defined by the first entry in preconfigComm defined in clause 6.8.1.1 (used for transmission by an out of coverage UE) should not overlap (see TS 36.331 [17], clause 9.3.2) with those of the pool(s) covering scheduled transmissions by in coverage UEs which are defined here for SIB18. |  |  |  |

| Condition | Explanation |
| --- | --- |
| BW5 | 5 MHz channel bandwidth cell environment |
| BW10 | 10 MHz channel bandwidth cell environment |
| FDD | FDD cell environment |

#### -	SystemInformationBlockType19

The IE SystemInformationBlockType19 contains the resource pool information for sidelink direct discovery.

Table 4.4.3.3-17: SystemInformationBlockType19

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType19-r12 ::= SEQUENCE { |  |  |  |
| discConfig-r12 SEQUENCE { |  |  |  |
| discRxPool-r12 SEQUENCE SIZE (1..maxSL-TxPool-r12) OF SL-DiscResourcePool-r12 { |  | 2 RxPools For convenience the difference between the pools is only in subframeBitmap. |  |
| SL-DiscResourcePool-r12[1] SEQUENCE { |  | RxPool 1 A monitoring UE can receive on the resources of this pool when a transmitting UE uses the TxPoolCommon 1 |  |
| cp-Len-r12 | normal |  |  |
| discPeriod-r12 | rf128 |  |  |
| numRetx-r12 | 0 |  |  |
| numRepetition-r12 | 1 |  |  |
| tf-ResourceConfig-r12 SEQUENCE { |  | SL-TF-ResourceConfig-r12 |  |
| prb-Num-r12 | 12 |  | BW5 |
|  | 24 |  | BW10 |
| prb-Start-r12 | 0 |  |  |
| prb-End-r12 | 23 |  | BW5 |
|  | 48 |  | BW10 |
| offsetIndicator-r12 | 160 | small-r12 | FDD |
|  | 163 |  | TDD |
| subframeBitmap-r12 | 11000000 00000000 00000000 00000000 00000000 | bs40-r12 | FDD |
|  | 11000000 00000000 | bs16-r12 | TDD |
| } |  |  |  |
| txParameters-r12 | Not present |  |  |
| rxParameters-r12 | Not present |  |  |
| } |  |  |  |
| SL-DiscResourcePool-r12[2] SEQUENCE { |  | RxPool 2 A monitoring UE can receive on the resources of this pool when a transmitting UE uses the TxPoolCommon 2 |  |
| cp-Len-r12 | normal |  |  |
| discPeriod-r12 | rf128 |  |  |
| numRetx-r12 | 0 |  |  |
| numRepetition-r12 | 1 |  |  |
| tf-ResourceConfig-r12 SEQUENCE { |  | SL-TF-ResourceConfig-r12 |  |
| prb-Num-r12 | 12 |  | BW5 |
|  | 24 |  | BW10 |
| prb-Start-r12 | 0 |  |  |
| prb-End-r12 | 23 |  | BW5 |
|  | 48 |  | BW10 |
| offsetIndicator-r12 | 160 | small-r12 | FDD |
|  | 163 |  | TDD |
| subframeBitmap-r12 | 00110000 00000000 00000000 00000000 00000000 | bs40-r12 | FDD |
|  | 00110000 00000000 | bs16-r12 | TDD |
| } |  |  |  |
| txParameters-r12 | Not present |  |  |
| rxParameters-r12 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| discTxPoolCommon-r12 SEQUENCE SIZE (1..maxSL-TxPool-r12) OF SL-DiscResourcePool-r12 { |  | 2 TxPools For convenience the difference between the pools is only in subframeBitmap. |  |
| SL-DiscResourcePool-r12[1] SEQUENCE { |  | TxPool 1 |  |
| cp-Len-r12 | normal |  |  |
| discPeriod-r12 | rf128 |  |  |
| numRetx-r12 | 0 |  |  |
| numRepetition-r12 | 1 |  |  |
| tf-ResourceConfig-r12 SEQUENCE { |  | SL-TF-ResourceConfig-r12 |  |
| prb-Num-r12 | 12 |  | BW5 |
|  | 24 |  | BW10 |
| prb-Start-r12 | 0 |  |  |
| prb-End-r12 | 23 |  | BW5 |
|  | 48 |  | BW10 |
| offsetIndicator-r12 | 160 | small-r12 | FDD |
|  | 163 |  | TDD |
| subframeBitmap-r12 | 11000000 00000000 00000000 00000000 00000000 | bs40-r12 | FDD |
| bs42-r12 | 11000000 00000000 | bs16-r12 | TDD |
| } |  |  |  |
| txParameters-r12 SEQUENCE { |  |  |  |
| txParametersGeneral-r12 SEQUENCE { |  | SL-TxParameters-r12 |  |
| alpha-r12 | al0 |  |  |
| p0-r12 | 31 | P0-SL-r12 |  |
| } |  |  |  |
| ue-SelectedResourceConfig-r12 SEQUENCE { |  |  |  |
| poolSelection-r12 | NULL | random-r12 |  |
| txProbability-r12 | p100 |  |  |
| } |  |  |  |
| } |  |  |  |
| rxParameters-r12 | Not present |  |  |
| } |  |  |  |
| SL-DiscResourcePool-r12[2] SEQUENCE { |  | TxPool 2 |  |
| cp-Len-r12 | normal |  |  |
| discPeriod-r12 | rf128 |  |  |
| numRetx-r12 | 0 |  |  |
| numRepetition-r12 | 1 |  |  |
| tf-ResourceConfig-r12 SEQUENCE { |  | SL-TF-ResourceConfig-r12 |  |
| prb-Num-r12 | 12 |  | BW5 |
|  | 24 |  | BW10 |
| prb-Start-r12 | 0 |  |  |
| prb-End-r12 | 23 |  | BW5 |
|  | 48 |  | BW10 |
| offsetIndicator-r12 | 160 | small-r12 | FDD |
|  | 163 |  | TDD |
| subframeBitmap-r12 | 00110000 00000000 00000000 00000000 00000000 | bs40-r12 | FDD |
|  | 00110000 00000000 | bs16-r12 | TDD |
| } |  |  |  |
| txParameters-r12 SEQUENCE { |  |  |  |
| txParametersGeneral-r12 SEQUENCE { |  | SL-TxParameters-r12 |  |
| alpha-r12 | al0 |  |  |
| p0-r12 | 31 | P0-SL-r12 |  |
| } |  |  |  |
| ue-SelectedResourceConfig-r12 SEQUENCE { |  |  |  |
| poolSelection-r12 | NULL | random-r12 |  |
| txProbability-r12 | p100 |  |  |
| } |  |  |  |
| } |  |  |  |
| rxParameters-r12 | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| discTxPowerInfo-r12 SEQUENCE (SIZE (maxSL-DiscPowerClass-r12)) OF SL-DiscTxPowerInfo-r12 { |  |  |  |
| SL-DiscTxPowerInfo-r12[1] SEQUENCE { |  | UE range class ‘short’ |  |
| discMaxTxPower-r12 | 23 |  |  |
| } |  |  |  |
| SL-DiscTxPowerInfo-r12[2] SEQUENCE { |  | UE range class ‘medium’ |  |
| discMaxTxPower-r12 | 16 |  |  |
| } |  |  |  |
| SL-DiscTxPowerInfo-r12[3] SEQUENCE { |  | UE range class ‘long’ |  |
| discMaxTxPower-r12 | 10 |  |  |
| } |  |  |  |
| } |  |  |  |
| discSyncConfig-r12 SEQUENCE (SIZE (1..maxSL-SyncConfig-r12)) OF SL-SyncConfig-r12 { | 2 configurations reception/transmission of synchronisation signals |  |  |
| SL-SyncConfig-r12[1] SEQUENCE { | SyncConfig 1 | To be used for transmission |  |
| syncCP-Len-r12 | normal |  |  |
| syncOffsetIndicator-r12 | 35 |  |  |
| slssid-r12 | 30 |  |  |
| txParameters-r12 SEQUENCE { |  |  |  |
| syncTxParameters-r12 SEQUENCE { |  | SL-TxParameters-r12 |  |
| alpha-r12 | al0 |  |  |
| p0-r12 | 31 |  |  |
| } |  |  |  |
| syncTxThreshIC-r12 | 0 | (-infinity) SLSS will not be transmitted |  |
| syncInfoReserved-r12 | Not Present |  |  |
| } |  |  |  |
| rxParamsNCell-r12 SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| SL-SyncConfig-r12[2] SEQUENCE { | SyncConfig 2 |  |  |
| syncCP-Len-r12 | normal |  |  |
| syncOffsetIndicator-r12 | 25 |  |  |
| slssid-r12 | 40 |  |  |
| txParameters-r12 SEQUENCE { |  |  |  |
| syncTxParameters-r12 SEQUENCE { |  | SL-TxParameters-r12 |  |
| alpha-r12 | al0 |  |  |
| p0-r12 | 4 |  |  |
| } |  |  |  |
| syncTxThreshIC-r12 | 7 | -85dBm The threshold for starting transmission of SLSS |  |
| syncInfoReserved-r12 | Not Present |  |  |
| } |  |  |  |
| rxParamsNCell-r12 SEQUENCE { |  |  |  |
| physCellId-r12 | Set to the physical cell identity of the PCell |  |  |
| discSyncWindow-r12 | w1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| discInterFreqList-r12 | Not present |  |  |
| lateNonCriticalExtension |  |  |  |
| ... |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment (TDD config 1) |
| BW5 | 5 MHz channel bandwidth cell environment |
| BW10 | 10 MHz channel bandwidth cell environment |

#### -	SystemInformationBlockType20

The IE SystemInformationBlockType20 contains the information required to acquire the SC-PTM control information.

Table 4.4.3.3-18: SystemInformationBlockType20

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType20-r13 ::= SEQUENCE { |  |  |  |
| sc-mcch-RepetionPeriod-r13 | Rf32 |  |  |
| sc-mcch-Offset-r13 | 1 |  |  |
| sc-mcch-FirstSubframe-r13 | 0 |  |  |
| sc-mcch-duration-r13 | Not present |  |  |
| sc-mcch-ModificationPeriod-r13 | Rf512 |  |  |
| lateNonCriticalExtension |  |  |  |
| } |  |  |  |

#### -	SystemInformationBlockType21

The IE SystemInformationBlockType21 contains the resource pool information for V2X sidelink communication.

Table 4.4.3.3-19: SystemInformationBlockType21

| Derivation Path: 36.331 clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType21-r14 ::= SEQUENCE { |  |  |  |
| sl-V2X-ConfigCommon-r14 SEQUENCE { |  |  |  |
| v2x-CommRxPool-r14 | Not present |  |  |
| v2x-CommTxPoolNormalCommon-r14 | Not present |  |  |
| p2x-CommTxPoolNormalCommon-r14 | Not present |  |  |
| v2x-CommTxPoolExceptional-r14 | Not present |  |  |
| v2x-SyncConfig-r14 | Not present |  |  |
| v2x-InterFreqInfoList-r14 SEQUENCE (SIZE (0..maxFreqV2X-1-r14)) OF SL-InterFreqInfoV2X-r14 { |  | 1 entry |  |
| SL-InterFreqInfoV2X-r14[1] | SL-InterFreqInfoV2X-r14-DEFAULT using condition SIB |  |  |
| } |  |  |  |
| v2x-ResourceSelectionConfig-r14 | Not present |  |  |
| zoneConfig-r14 SEQUENCE {} | Not present |  |  |
| typeTxSync-r14 | enb | ENUMERATED {gnss, enb, ue, spare1} |  |
| thresSL-TxPrioritization-r14 | Not present |  |  |
| anchorCarrierFreqList-r14 | Not present |  |  |
| offsetDFN-r14 | Not present |  |  |
| cbr-CommonTxConfigList-r14 | Not present |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

#### -	SystemInformationBlockType24

The IE SystemInformationBlockType24 contains information relevant only for inter-RAT cell re-selection i.e. information about NR frequencies and NR neighbouring cells relevant for cell re-selection. The IE includes cell re-selection parameters common for a frequency.

Table 4.4.3.3-20: SystemInformationBlockType24

| Derivation Path: TS 36.331 [17], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType24-r15 ::= SEQUENCE { |  |  |  |
| carrierFreqListNR-r15 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | The same number of entries as the configured NR carriers For Signalling test cases, see Table 6.3.1.8-1. | n denotes the index of the entry |  |
| carrierFreq-r15[n] | Downlink NR ARFCN of SSB under test | For signalling test cases see TS 38.508-1 [67] subclause 6.2.3. Otherwise, see TS 38.508-1 [67] subclause 4.3.1. |  |
| multiBandInfoList-r15 [n] SEQUENCE (SIZE (1..maxMultiBandsNR-r15)) OF SEQUENCE {} | Not present |  |  |
| multiBandInfoListSUL-r15[n] SEQUENCE (SIZE (1..maxMultiBandsNR-r15)) OF SEQUENCE {} | Not present |  |  |
| measTimingConfig-r15[n] SEQUENCE { |  |  |  |
| periodicityAndOffset-r15 CHOICE { |  |  |  |
| sf20 | 0 |  | FR1 |
| sf160 | 0 |  | FR2 |
| } |  |  |  |
| ssb-Duration-r15 | sf2 |  | FR1 |
|  | sf3 |  | FR2 |
| } |  |  |  |
| subcarrierSpacingSSB-r15[n] | kHz15 |  | SCS_15kHz |
|  | kHz30 |  | SCS_30kHz |
|  | kHz120 |  | SCS_120kHz |
| ss-RSSI-Measurement-r15[n] | Not present |  |  |
| cellReselectionPriority-r15[n] | 5 |  |  |
| cellReselectionSubPriority-r15[n] | Not present |  |  |
| threshX-High-r15[n] | 2 (4 dB) |  |  |
| threshX-Low-r15[n] | 1 (2 dB) |  |  |
| threshX-Q-r15[n] | Not present |  |  |
| threshX-Q-r15[n] SEQUENCE { |  |  | QBASED |
| threshX-HighQ-r15 | 5 (5 dB) |  |  |
| threshX-LowQ-r15 | 5 (5 dB) |  |  |
| } |  |  |  |
| q-RxLevMin-r15[n] | -70 (-140 dBm) | For RF/RRM test cases |  |
|  | -55 (-110 dBm) | For signalling test cases |  |
| q-RxLevMinSUL-r15[n] | -70 (-140 dBm) | For RF/RRM test cases | SUL |
|  | -55(-110 dBm) | For signalling test cases |  |
| p-MaxNR-r15[n] | 23 |  | FR1 |
|  | 26 |  | FR2 FR1_RF_PC2 |
| ns-PmaxListNR-r15[n] | Not present |  |  |
| q-QualMin-r15[n] | Not present |  |  |
|  | -20 (-20dB) |  | QBASED |
| deriveSSB-IndexFromCell-r15[n] | False |  |  |
| maxRS-IndexCellQual-r15[n] | Not present |  |  |
| threshRS-Index-r15[n] | Not present |  |  |
| } |  |  |  |
| t-ReselectionNR-r15 | 0 |  |  |
| t-ReselectionNR-SF-r15 | Not present | Not required unless speed-dependent cell re-selection is tested. |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SCS_15kHz | SCS=15kHz for frequency of the cell according to TS 38.508-1 [67] clause 6.2.3 for signalling test cases and clause 4.3.1 otherwise |
| SCS_30kHz | SCS=30kHz for frequency of the cell according to TS 38.508-1 [67] clause 6.2.3 for signalling test cases and clause 4.3.1 otherwise |
| SCS_120kHz | SCS=120kHz for frequency of the cell according to TS 38.508-1 [67] clause 6.2.3 for signalling test cases and clause 4.3.1 otherwise |
| QBASED | This condition applies to Quality based cell (re)selection signalling test cases. |
| FR1_RF_PC2 | FR1 RF testing with Power Class 2 |

#### 4.4.3.4	Channel-bandwidth-dependent parameters in system information blocks

The default values of parameters in system information blocks which depend on the channel bandwidth are defined in table 4.4.3.4-1.

Table 4.4.3.4-1: Channel-bandwidth-dependent parameters

| Information Element | Channel bandwidth |  |  |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20MHz |  |
| SIB3 periodicity | 64 | 64 | 32 | 32 | 32 | 32 |  |
| SIB4 periodicity | 128 | 128 | 64 | 64 | 64 | 64 |  |
| SIB5 periodicity | 128 | 128 | 64 | 64 | 64 | 64 |  |
| SIB6 periodicity | 128 | 128 | 64 | 64 | 64 | 64 |  |
| SIB7 periodicity | 128 | 128 | 64 | 64 | 64 | 64 |  |
| SIB8 periodicity | 128 | 128 | 64 | 64 | 64 | 64 |  |
| measurementBandwidth in SIB5 | mbw6 | mbw15 | mbw25 | mbw50 | mbw75 | mbw100 |  |
| SIB9 periodicity | 128 | 128 | 64 | 64 | 64 | 64 |  |
| SIB10 periodicity | 64 | 64 | 32 | 32 | 32 | 32 |  |
| SIB11 periodicity | 64 | 64 | 32 | 32 | 32 | 32 |  |
| SIB12 periodicity | 64 | 64 | 32 | 32 | 32 | 32 |  |
| SIB13 periodicity | 64 | 64 | 32 | 32 | 32 | 32 |  |
| SIB15 periodicity | 64 | 64 | 32 | 32 | 32 | 32 |  |
| SIB17 periodicity | 128 | 128 | 64 | 64 | 64 | 64 |  |
| SIB18 periodicity |  |  | 32 | 32 | - | 32 |  |
| SIB19 periodicity |  |  | 64 | 64 | 64 | 64 |  |
| SIB20 periodicity | 64 | 64 | 32 | 32 | 32 | 32 |  |
| SIB21 periodicity |  |  |  | 32 |  | 32 |  |
| SIB24 periodicity | 128 | 128 | 64 | 64 | 64 | 64 |  |

### 4.4.4	Common parameters for simulated UTRA cells

The parameters specified in this subclause apply to all simulated UTRA cells unless otherwise specified.

Default UTRA parameters for simulated cells are specified in table 4.4.4-1 and table 4.4.4-2.

Other parameters are specified in TS 34.108 [5].

Table 4.4.4-1: Default parameters for simulated UTRA cells

| cell ID | Primary scrambling code for FDD | Cell parameters ID for TDD |
| --- | --- | --- |
| Cell 5 | 100 | 0 |
| Cell 7 | 150 | 4 |
| Cell 8 | 200 | 8 |
| Cell 9 | 250 | 12 |
| Cell 37 | 300 | 16 |
| Cell 38 | 350 | 20 |
| Cell 39 | 400 | 24 |

Table 4.4.4-2: Default NAS parameters for simulated UTRA cells

|  | PLMN |  | Location Area Identification |  | Routing Area Identification |  | TMSI | P-TMSI signature |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MCC | MNC | LA# | LAC | RA# | RAC |  |  |
| Cell 5 | (Note 1) |  | LAI-1 | 1 | RAI-1 | 1 | Arbitrarily selected according to TS 23.003 subclause 2.4 [2] | Arbitrarily selected according to TS 23.003 subclause 2.7 [2] |
| Cell 7 | (Note 1) |  | LAI-1 | 1 | RAI-1 | 1 |  |  |
| Cell 8 | (Note 1) |  | LAI-1 | 1 | RAI-1 | 1 |  |  |
| Cell 9 | (Note 1) |  | LAI-1 | 1 | RAI-1 | 1 |  |  |
| Cell 37 | (Note 1) |  | LAI-1 | 1 | RAI-1 | 1 |  |  |
| Cell 38 | (Note 1) |  | LAI-1 | 1 | RAI-1 | 1 |  |  |
| Cell 39 | (Note 1) |  | LAI-1 | 1 | RAI-1 | 1 |  |  |
| Note 1: Set to the same Mobile Country Code and Mobile Network Code stored in EFIMSI on the test USIM card (subclause 4.9.3). |  |  |  |  |  |  |  |  |

#### 4.4.4.1	Common contents of system information blocks for UTRA cells

#### -	System Information Block type 19

The system information block type 19 contains Inter-RAT frequency and priority information to be used in the cell.

Table 4.4.4.1-1: System Information Block type 19

| Derivation Path: 25.331 clause 11.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList SEQUENCE { |  |  |  |
| utra-ServingCell SEQUENCE { |  |  |  |
| priority | 3 |  |  |
| s-PrioritySearch1 | 0 (0dB) |  |  |
| s-PrioritySearch2 | Not present | default value is 0 |  |
| threshServingLow | 0 (0dB) |  |  |
| } |  |  |  |
| utran-FDD-FrequencyList SEQUENCE (SIZE(1..maxNumFDDFreqs)) OF SEQUENCE {} | Not present |  |  |
| utran-TDD-FrequencyList SEQUENCE (SIZE(1..maxNumTDDFreqs)) OF SEQUENCE {} | Not present |  |  |
| gsm-PriorityInfoList SEQUENCE (SIZE (1..maxNumGSMCellGroup)) OF SEQUENCE {} | Not present |  |  |
| eutra-FrequencyAndPriorityInfoList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { | The same number of entries as the configured eutra carriers For Signalling test cases, see table 6.3.1.7-1 | n denotes the index of the entry |  |
| earfcn[n] | Downlink EARFCN under test For Signalling test cases, see table 6.3.1.7-1 |  |  |
| measurementBandwidth[n] | Set according to EUTRA band ChBw in Table 5.6-1 of TS36.104. |  |  |
| priority[n] | 4 |  |  |
| qRxLevMinEUTRA[n] | -53 (-106 dBm) | For signalling test cases |  |
|  | -70 (-140 dBm) | For RF/RRM test cases |  |
| threshXhigh[n] | 2 (4 dB) |  |  |
| threshXlow[n] | 1 (2 dB) |  |  |
| eutra-blackListedCellList[n] | Not present |  |  |
| eutraDetection[n] | TRUE |  |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList | Not present |  | Band > 64 |
| v920NonCriticalExtensions SEQUENCE { |  |  | QBASED |
| va80NonCriticalExtensions SEQUENCE { |  |  | Band > 64 |
| vb30NonCriticalExtensions SEQUENCE { |  |  |  |
| vb50NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-vb50ext SEQUENCE { |  |  |  |
| numberOfApplicableEARFCN | 0 |  |  |
| eutra-FrequencyAndPriorityInfoExtensionList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |
| earfcn | Downlink EARFCN under test For Signalling test cases, see table 6.3.1.7-1 |  |  |
| measurementBandwidth | Set according to EUTRA band ChBw in Table 5.6-1 of TS 36.104. |  |  |
| priority | 4 |  |  |
| qRxLevMinEUTRA | -53 (-106 dBm) | For signalling test cases |  |
|  | -70 (-140 dBm) | For RF/RRM test cases |  |
| threshXhigh | 2 (4 dB) |  |  |
| threshXlow | 1 (2 dB) |  |  |
| qqualMinEUTRA | Not present |  |  |
| threshXhigh2 | Not present |  |  |
| threshXlow2 | Not present |  |  |
| eutra-blackListedCellList | Not present |  |  |
| eutraDetection | TRUE |  |  |
| } |  |  |  |
| } |  |  |  |
| vb80NonCriticalExtensions SEQUENCE {} |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| sysInfoType19-v920ext SEQUENCE { |  |  |  |
| utra-PriorityInfoList-v920ext SEQUENCE { |  |  |  |
| threshServingLow2 | 7 (7dB) | default value is 0 |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList-v920ext (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |
| qQualMinEUTRA[n] | -20 (-20dB) |  |  |
| threshXhigh2[n] | 5 (5dB) |  |  |
| threshXlow2[n] | 5 (5dB) |  |  |
| } |  |  |  |
| v920NonCriticalExtensions SEQUENCE { |  |  | Band > 64 |
| va80NonCriticalExtensions SEQUENCE { |  |  |  |
| vb30NonCriticalExtensions SEQUENCE { |  |  |  |
| vb50NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-vb50ext SEQUENCE { |  |  |  |
| numberOfApplicableEARFCN | 0 |  |  |
| eutra-FrequencyAndPriorityInfoExtensionList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |
| earfcn | Downlink EARFCN under test For Signalling test cases, see table 6.3.1.7-1 |  |  |
| measurementBandwidth | Set according to EUTRA band ChBw in Table 5.6-1 of TS 36.104. |  |  |
| priority | 4 |  |  |
| qRxLevMinEUTRA | -53 (-106 dBm) | For signalling test cases |  |
|  | -70 (-140 dBm) | For RF/RRM test cases |  |
| threshXhigh | 2 (4 dB) |  |  |
| threshXlow | 1 (2 dB) |  |  |
| qqualMinEUTRA | Not present |  |  |
| threshXhigh2 | Not present |  |  |
| threshXlow2 | Not present |  |  |
| eutra-blackListedCellList | Not present |  |  |
| eutraDetection | TRUE |  |  |
| } |  |  |  |
| } |  |  |  |
| vb80NonCriticalExtensions SEQUENCE {} |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtensions SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| QBASED | See the definition below table 4.4.3.2-3. |
| Band > 64 | If band > 64 is selected |

#### 4.4.4.2	UTRA SIB scheduling for inter EUTRA - UTRA test

Table 4.4.4.2-1: UTRA SIB schedule

| Block Type | MIB | SB1 | SIB1 | SIB2 | SIB3 | SIB4 | SIB5/SIB5bis | SIB6 | SIB7 | SIB11 | SIB12 | SIB18 | SIB19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SIB_REP | 8 | 16 | 64 | 64 | 64 | 64 | 64 | 64 | 16 | 64 | 64 | 64 | 64 |
| SEG_ COUNT | 1 | 1 | 1 | 1 | 1 | 1 | 4 | 4 | 1 | 3 | 1 | 1 | 2 |

Table 4.4.4.2-2: UTRA SIB-repeat period

| Frame No / SIB_POS | 0 | 2 | 4 | 6 | 8 | 10 | 12 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Block Type | MIB | SB1 | SIB7 | SIB6 | MIB | SIB6 | SIB6 | SIB6 |
| Frame No / SIB_POS | 16 | 18 | 20 | 22 | 24 | 26 | 28 | 30 |
| Block Type | MIB | SB1 | SIB7/SIB3 | SIB1/SIB2 | MIB | SIB12 | SIB19 | SIB19 |
| Frame No / SIB_POS | 32 | 34 | 36 | 38 | 40 | 42 | 44 | 46 |
| Block Type | MIB | SB1 | SIB7/SIB18 | SIB5/ SIB5bis | MIB | SIB5/ SIB5bis | SIB5/ SIB5bis | SIB5/ SIB5bis |
| Frame No / SIB_POS | 48 | 50 | 52 | 54 | 56 | 58 | 60 | 62 |
| Block Type | MIB | SB1 | SIB7/SIB4 | - | MIB | SIB11 | SIB11 | SIB11 |

#### 4.4.4.3	UTRA SIB scheduling for inter EUTRA – UTRA - GERAN test

Table 4.4.4.3-1: UTRA SIB schedule

| Block Type | MIB | SB1 | SIB1 | SIB2 | SIB3 | SIB4 | SIB5/ SIB5bis | SIB7 | SIB11 | SIB16 | SIB18 | SIB19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SIB Rep | 8 | 16 | 128 | 128 | 64 | 64 | 128 | 32 | 128 | 128 | 128 | 128 |
| Max. No of seg. | 1 | 2 | 1 | 1 | 1 | 1 | 4 | 1 | 3 | 8 | 1 | 3 |

Table 4.4.4.3-2: UTRA SIB-repeat period

| Frame No / SIB_POS | 0 | 2 | 4 | 6 | 8 | 10 | 12 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Block Type | MIB | SB1 | SB1 | - | MIB | SIB1 | SIB18 | SIB2 |
| Frame No / SIB_POS | 16 | 18 | 20 | 22 | 24 | 26 | 28 | 30 |
| Block Type | MIB | SB1 | SB1 | SIB7 | MIB | SIB3 | - | SIB4 |
| Frame No / SIB_POS | 32 | 34 | 36 | 38 | 40 | 42 | 44 | 46 |
| Block Type | MIB | SB1 | SB1 | SIB5/ SIB5bis | MIB | SIB5/ SIB5bis | SIB5/ SIB5bis | SIB5/ SIB5bis |
| Frame No / SIB_POS | 48 | 50 | 52 | 54 | 56 | 58 | 60 | 62 |
| Block Type | MIB | SB1 | SB1 | SIB7 | MIB | SIB11 | SIB11 | SIB11 |
| Frame No / SIB_POS | 64 | 66 | 68 | 70 | 72 | 74 | 76 | 78 |
| Block Type | MIB | SB1 | SB1 | SIB16 | MIB | SIB16 | SIB16 | SIB16 |
| Frame No / SIB_POS | 80 | 82 | 84 | 86 | 88 | 90 | 92 | 94 |
| Block Type | MIB | SB1 | SB1 | SIB7 | MIB | SIB3 | SIB19 | SIB4 |
| Frame No / SIB_POS | 96 | 98 | 100 | 102 | 104 | 106 | 108 | 110 |
| Block Type | MIB | SB1 | SB1 | SIB16 | MIB | SIB16 | SIB16 | SIB16 |
| Frame No / SIB_POS | 112 | 114 | 116 | 118 | 120 | 122 | 124 | 126 |
| Block Type | MIB | SB1 | SB1 | SIB7 | MIB | SIB19 | SIB19 | - |

### 4.4.5	Common parameters for simulated GERAN cells

The parameters specified in this subclause apply to all simulated GERAN cells unless otherwise specified.

See TS 51.010 [25].

The contents of SYSTEM INFORMATION TYPE 2 Quarter message specified below are identical for all GERAN cells.

Table 4.4.5-1: System Information 2 Quater

| Derivation Path: 44.018 Table 10.5.2.33b.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| System information 2Quater |  |  |  |
| L2 pseudo length | 1 |  |  |
| PROTOCOL DISCRIMINATOR | ‘0110’B | RR management Protocol |  |
| SKIP INDICATOR | ‘0000’B |  |  |
| MESSAGE TYPE | ‘00000111’B |  |  |
| SI 2QUATER REST OCTETS |  |  |  |
| BA Indicator | 0 |  |  |
| 3G BA Indicator | 0 |  |  |
| MP Change Mark | 0 |  |  |
| SI2Q Index | 0 |  |  |
| SI2Q Count | 0 |  |  |
| Measurement_Parameters Description | 0 | Not Present |  |
| GPRS_Real Time Difference Description | 0 | Not Present |  |
| GPRS_BSIC Description | 0 | Not Present |  |
| GPRS_REPORT PRIORITY Description | 0 | Not Present |  |
| GPRS_MEASUREMENT_Parameters Description | 0 | Not Present |  |
| NC Measurement Parameters | 0 | Not Present |  |
| extension length | 0 | Not Present |  |
| 3G Neighbour Cell Description | 0 | Not Present |  |
| 3G Measurement Parameters Description | 0 | Not Present |  |
| GPRS_3G_MEASUREMENT Parameters Description | 0 | Not Present |  |
| Additions in release 5 | H |  |  |
| 3G Additional Measurement Parameters Description | 0 | Not Present |  |
| 3G ADDITIONAL MEASUREMENT Parameters Description 2 | 0 | Not Present |  |
| Additions in release 6 | H |  |  |
| 3G_CCN_ACTIVE | 0 |  |  |
| Additions in release 7 | H |  |  |
| 700_REPORTING_OFFSET | 0 | Not Present |  |
| 810_REPORTING_OFFSET | 0 | Not Present |  |
| Additions in release 8 | H |  |  |
| Priority and E-UTRAN Parameters Description | 1 | Present |  |
| Serving Cell Priority Parameters Description | 1 | Present Note: Values as defined from TS 45.008, section 10.4 Table 3 |  |
| GERAN_PRIORITY | 2 |  |  |
| THRESH_Priority_Search | 15 (always) |  |  |
| THRESH_GSM_low | 0 |  |  |
| H_PRIO | 0 |  |  |
| T_Reselection | 0 |  |  |
| 3G Priority Parameters Description | 0 | Not Present |  |
| E-UTRAN Parameters Description | 1 | Present |  |
| E-UTRAN_CCN_ACTIVE | 0 | CCN not supported |  |
| E-UTRAN_Start | 1 |  |  |
| E-UTRAN_Stop | 1 |  |  |
| E-UTRAN Measurement Parameters Description | 0 | Not Present |  |
| GPRS E-UTRAN Measurement Parameters Description | 0 | Not Present |  |
| Repeated E-UTRAN Neighbour Cells { | 1 | Present |  |
| EARFCN | Same as cell 1 | This field specifies the E-UTRA Absolute Radio Frequency Channel Number as defined in 3GPP TS 36.104. |  |
| EARFCN | 65535 |  | Band > 64 |
| Measurement Bandwidth | Set according to EUTRA band under test |  |  |
| E-UTRAN_PRIORITY | ‘100’B | Same as cell 1 as defined in 3GPP TS 36.104. |  |
| THRESH_E-UTRAN_high | ‘00010’B | Actual value =4 dB |  |
| THRESH_E-UTRAN_low | ‘00010’B | Actual value =4 dB |  |
| E-UTRAN_QRXLEVMIN | 17 (-106 dBm) | For signalling test cases |  |
|  | 0 (-140 dBm) | For RF/RRM test cases |  |
| } ** 0 |  | End of Repeated E-UTRAN Neighbour cells structure. Note: When this message is used in Tics where more than one E-UTRA cells are active, this structure needs to be specified for each cell. EARFCN and EUTRAN_PRIORITY will then need to be configured to the corresponding EUTRAN cell |  |
| Repeated E-UTRAN Not Allowed Cells | 0 | Not Present |  |
| Repeated E-UTRAN PCID to TA mapping | 0 | Not Present |  |
| 3G CSG Description | 0 |  |  |
| E-UTRAN CSG Description | 0 |  |  |
| Additions in release 9 | L |  |  |
| Additions in release 9 | H |  | Band > 64 |
| Enhanced Cell Reselection Parameters | 0 |  |  |
| CSG Cells Reporting Description | 0 |  |  |
| Additions in release 10 | H |  |  |
| INIT_PWR_RED | 0 |  |  |
| NC2_CSG_PCCN_permitted | 0 |  |  |
| Additions in release 11 | H |  |  |
| Extended EARFCNs Description | 1 |  |  |
| Repeated EARFCN_extended { | 1 | Present |  |
| EARFCN_extended | Same as cell 1 | This field specifies the E-UTRA Absolute Radio Frequency Channel Number as defined in 3GPP TS 36.104. |  |
| } ** 0 |  | End of Repeated EARFCN_extended |  |
| Extended EARFCNs Description for CSG Cells | 0 |  |  |
| spare padding | spare padding |  |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

### 4.4.6	Common parameters for simulated CDMA2000 cells

The parameters specified in this subclause apply to all simulated HRPD or 1xRTT cells unless otherwise specified.

See C.S0038-B [36] for HRPD cells and C.S0043-A [37] for 1xRTT cells.

### 4.4.7	Default parameters specific for simulated cells

Default parameters specific for simulated cells are specified in this subclause.

#### 4.4.7.1	Common contents of HRPD Overhead messages

-	QuickConfig

The QuickConfig message is used to indicate a change in the overhead messages’ contents and to provide frequently changing information.

Table 4.4.7.1-1: QuickConfig

| Derivation Path: C.S0024-B clause 7.11.6.2.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MessageID | 0x0 | 0-255; |  |
| ColorCode | See table 4.4.2-3 | 0 – 255 |  |
| SectorID24 | Least significant 24 bits of Sector ID given in table 4.4.2-3 | 24 bits |  |
| SectorSignature | A valid value and same as used for SectorParameters | 16 bits |  |
| AccessSignature | A valid value and same as used for AccessParameters | 16 bits |  |
| Redirect | 0X0 | 0-1 |  |
| RPCCount63To0 | A valid value corresponding to the value of px_MACIndex | 0-63 |  |
| ForwardTrafficValid63To0 | A valid value corresponding to the value of px_MACIndex | 0-1; RPCCount63To0 occurrences |  |
| RPCCount127To64Included | A valid value corresponding to the value of px_MACIndex | 0 or 1 bits |  |
| RPCCount127To64 | A valid value corresponding to the value of px_MACIndex | 0 or 6 bits |  |
| ForwardTrafficValid127To64 | A valid value corresponding to the value of px_MACIndex | 0 or 1 bits;RPCCount127To64 occurrences |  |
| RPCCount130To383Included | Not Present or a valid value corresponding to the value of px_MACIndex | 0 or 1 bits |  |
| RPCCount130To383 | Not Present or a valid value corresponding to the value of px_MACIndex | 0 or 8 bits |  |
| ForwardTrafficValid130To383 | Not Present or a valid value corresponding to the value of px_MACIndex | 0 or 1 bits |  |
| Reserved | 0X0 | 0 – 7 bits as needed all set to zero |  |

-	SectorParameters

The SectorParameters message is used to convey sector specific information to the access terminals.

Table 4.4.7.1-2: SectorParameters

| Derivation Path: C.S0024-B clause 7.11.6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MessageID | 0x01 | 0-255 |  |
| CountryCode | See table 4.4.2-3 | 0 – 999;Depends on test case |  |
| SectorID | See table 4.4.2-3 | 128 bit value |  |
| SubnetMask | 0x40 | 8 bits |  |
| SectorSignature | Any valid value | 16 bits |  |
| Latitude | Any valid value | 22 bits |  |
| Longitude | Any valid value | 23 bits |  |
| RouteUpdateRadiusOverhead | 0x0 | 11 bits |  |
| LeapSeconds | 'A valid value, calculated and updated by the SS' | 8 bits |  |
| LocalTimeOffset | Any valid value | 11 bits |  |
| ReverseLinkSilenceDuration | 0x1 or 0x0 | 2 bits |  |
| ReverseLinkSilencePeriod | 0x1 or 0x0 | 2 bits |  |
| ChannelCount | 0x1 or 0x0 | 5 bits |  |
| Channel |  | 24 bits; ChannelCount occurrences; Sub-clause 13.1 in 3GPP2 C.S0024-B v3.0 |  |
| SystemType | 0x00 | 8 bits |  |
| BandClass | Set according to PIXIT parameter | 5 bits; Depends on test case |  |
| ChannelNumber | Set according to PIXIT parameter | 11 bits; Depends on BandClass |  |
| NeighborCount | 0x0 | 5 bits | No_NeighPNInc |
|  | 16 |  | NeighPNInc |
| NeighborPilotPN | Not Present |  | No_NeighPNInc |
|  | Pilots PN's of configured cells [FFS] |  | NeighPNInc |
| NeighborChannelIncluded | 0x0 | 1 bit; NeighbourCount occurrences | NeighPNInc |
|  | Not Present |  | No_NeighPNInc |
| NeighborChannel | Not Present | 0 or 24 bits; NeighbourCount occurrences |  |
| NeighborSearchWindowSizeIncluded | 0x0 | 1 bit |  |
| NeighborSearchWindowSize | Not Present | 0 or 4 bits; NeighbourCount occurrences |  |
| NeighborSearchWindowOffsetIncluded | 0x0 | 1 bit |  |
| NeighborSearchWindowOffset | Not Present | 0 or 3 bits; NeighbourCount occurrences |  |
| ExtendedChannelIncluded | 0x0 or Not Present | 0 or 1 bits; The number of cdma2000 high rate packet data extended channels available to the access terminal on this sector; Note 1; |  |
| ExtendedChannelCount | Not Present | 0 or 5 bits |  |
| ExtendedChannel | Not Present | 24 bits; 0 or ExtendedChannelCount occurrences |  |
| AccessHashingChannelMaskIncluded | 0x0 or Not Present | 0 or 1 bits; AccessHashingChannelMaskLength occurrences of this field and AccessHashingChannelMask if set as AccessHashingChannelMask; Note 1; |  |
| AccessHashingMaskLength | Not Present | 0 or 4 bits; 0<=AccessHashingChannelMaskLength<=(ChannelCount + ExtendedChannelCount), |  |
| AccessHashingChannelMaskSameAsPrevious | Not Present | 1 bit; AccessHashingChannelMaskLength occurrences of this field and AccessHashingChannelMask if set as AccessHashingChannelMask |  |
| AccessHashingChannelMask | Not Present | 0 or AccessHashingMaskLength+1 |  |
| RouteUpdateTriggerCodeIncluded | 0x0 or Not Present | 0 or 1 bits; Note 1; |  |
| RouteUpdateTriggerCode | Not Present | 0 or 12 bits |  |
| RouteUpdateTriggerMaxAge | Not Present | 0 or 4 bits |  |
| PriorSessionGAUP | 0x0 or Not Present | 0 or 1 bits; Note 1; |  |
| FPDCHSupportedInclude | 0x0 or Not Present | 0 or 1 bits; Note 1; |  |
| FPDCHSupported | Not Present | 0 or 1 bits |  |
| SecondaryColorCodeIncluded | 0x0 or Not Present | 0 or 1 bits; Note 1; |  |
| SecondaryColorCodeCount | Not Present | 0 or 3 bits |  |
| SecondaryColorCode | Not Present | 8 bits |  |
| PilotGroupIDLoopIncluded | 0x0 or Not Present | 0 or 1 bits; Note 1; |  |
| PilotGroupIDIncluded | Not Present | 0 or 1 bits |  |
| PilotGroupID | Not Present | 0 or 3 bits |  |
| IsSectorMultiCarrierCapable | 0x0 or Not Present | 0 or 1 bits; Note 1; |  |
| ReverseBandClass | Not Present | 0 or 5 bits |  |
| ReverseChannelNumber | Not Present | 0 or 11 bits |  |
| Reserved | 0X0 | 0 – 7 bits as needed all set to zero |  |
| Note 1: As no Non reserved fields following this are included, it can be Not Present; If not present, AT assumes as 0x0. |  |  |  |

| Condition | Explanation |
| --- | --- |
| No_NeighPNInc | No neighbour Pilot PN included |
| NeighPNInc | Neighbor Pilot PN's included |

-	AccessParameters

The AccessParameters message is used to convey Access Channel information to the access terminals.

Table 4.4.7.1-3: AccessParameters

| Derivation Path: C.S0024-B clause 9.4.6.2.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MessageID | 0x1 | 0-255 |  |
| Access Cycle Duration | 16 slots or 64 slots | 16,12,64,128 slots |  |
| Access Signature | Any valid value | 16 bits Note1. |  |
| OpenLoopAdjust | Set according to PIXIT parameter for default open loop adjust | 8bits(-84 – 115 dB); Actual value is -1 times |  |
| ProbeInitialAdjust | 0x0(0 dB) | 5 bits(-16 – 15 db) |  |
| ProbeNumStep | 0x5 | 4bits(1 – 15) |  |
| PowerStep | 0x8(4 dB) | 4 bits(0 – 7.5 dB) |  |
| PreambleLength | 0x2 | 3bits(1 – 7) |  |
| CapsuleLengthMax | 0x2 or 0x4 | 4 bits |  |
| APersistence | 0x0 or 0x1 | 6 bits; 'NACMPAPersist = 4' occurrences |  |
| APersistence | 0x0 or 0x1 | 6 bits |  |
| APersistence | 0x0 or 0x1 | 6 bits |  |
| APersistence | 0x0 or 0x1 | 6 bits |  |
| Reserved | 0X0 | 0 – 7 bits as needed all set to zero |  |
| Note 1: The value specified is the value to be used when the AccessParameters is being sent first time. At every change of content of AccessParameters message it is incremented by 1. |  |  |  |

-	Sync

The access network broadcasts the Sync message to convey basic network and timing information

Table 4.4.7.1-4: Sync

| Derivation Path: C.S0024-B clause 7.3.6.2.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MessageID | 0x0 | 0-255 |  |
| MaximumRevision | 0x1 | 8 bits ; Clause 1.15 of 3GPP2 C.S0024-B v3.0 |  |
| MinimumRevision | 0x1 | 8 bits; Clause 1.15 of 3GPP2 C.S0024-B v3.0 |  |
| Pilot PN Offset | See table 4.4.2-3 | 0 – 511 |  |
| CDMA System Time | A valid value, calculated and updated by the SS | 37 bits |  |

#### 4.4.7.2	Common contents of 1XRTT Overhead messages

##### 4.4.7.2.1	Configuration sequence number

In general, a base station maintains a configuration sequence number CONFIG_SEQ for configuration messages transmitted on the Paging channel and increments by 1 modulo 64 whenever any of the following messages being transmitted are modified:

System Parameters Message

Neighbor List Message (Band Class 0 only)

CDMA Channel List Message

Extended System Parameters Message

Extended Neighbor List Message (band classes other than Band Class 0).

For simplicity resaons the System Simulator may use any value for this configuration sequence number CONFIG_SEQ.

##### 4.4.7.2.2	Over Head messages

-	CDMA Channel List Message

Table 4.4.7.2.2-1: CDMA Channel List Message

| Derivation Path: C.S0005 clause 3.7.2.3.2.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PILOT_PN | See table 4.4.2-4 | 9 bits; |  |
| CONFIG_MSG_SEQ | Set to ' CONFIG_SEQ' of base station as per clause 4.4.7.2.1 | 6 bits; |  |
| CDMA_FREQ | Set according to PIXIT parameter or as per clause 4.3.1.4 | 11 bits |  |

-	System Parameters Message

Table 4.4.7.2.2-2: System Parameters Message

| Derivation Path: C.S0005 clause 3.7.2.3.2.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PILOT_PN | See table 4.4.2-4 | 9 bits |  |
| CONFIG_MSG_SEQ | Set to ' CONFIG_SEQ' of base station as per clause 4.4.7.2.1 | 6 bits |  |
| SID | See table 4.4.2-4 | 15 bits |  |
| NID | See table 4.4.2-4 | 16 bits |  |
| REG_ZONE | See table 4.4.2-4 | 12 bits |  |
| TOTAL_ZONES | 1 | 3 bits |  |
| ZONE_TIMER | 0 or 1 | 3 bits; 1 or 2 minutes |  |
| MULT_SIDS | 0 or 1 | 1 bits; Mobile shall not or shall store SID_NID_List containing different SID's |  |
| MULT_NIDS | 0 or 1 | 1 bits; Mobile shall not or shall store SID_NID_List containing different NID's |  |
| BASE_ID | See table 4.4.2-4 | 16 bits |  |
| BASE_CLASS | 1 | 4 bits | BandClass1_4 |
|  | 0 |  | Not BandClass1_4 |
| PAGE_CHAN | 1 | 3 bits; Number of Paging Channels |  |
| MAX_SLOT_CYCLE_INDEX | 1 or 0 | 3 bits; Maximum Slot Cycle index |  |
| HOME_REG | 1 | 1 bits; Home registration enabled |  |
| FOR_SID_REG | 1 | 1 bits; SID roamer registration enabled |  |
| FOR_NID_REG | 1 | 1 bits; NID roamer registration enabled |  |
| POWER_UP_REG | 1 | 1 bits; power-up registration enabled |  |
| POWER_DOWN_REG | Set according to PIXIT parameter | 1 bits; power-down registration enabled as per PIXIT |  |
| PARAMETER_REG | 0 or 1 | 1 bits; parameter change registration disabled or enabled |  |
| REG_PRD | 0 | 7 bits; registration period; No timer Based Registration |  |
| BASE_LAT | Any valid value | 22 bits; Base Station Latitude |  |
| BASE_LONG | Any valid value | 23 bits; Base Station Longitude |  |
| REG_DIST | 0 | 11 bits; Distance based registration disabled |  |
| SRCH_WIN_A | 8 | 4 bits; Search Window size for Active Set; 60 PN Chips |  |
| SRCH_WIN_N | 8 | 4 bits; Search Window Size for the neighbor set; 60 PN Chips |  |
| SRCH_WIN_R | 8 | 4 bits; Search window size for the remaining set; 60 PN Chips |  |
| NGHBR_MAX_AGE | 0 | 4 bits; Minimum Neighbour set age |  |
| PWR_REP_THRESH | 0, 1 or 5 | 5 bits; Power control reporting threshold |  |
| PWR_REP_FRAMES | 0, 4 or 7 | 4 bits; power control reporting frame count; 5 or 28 frames |  |
| PWR_THRESH_ENABLE | 0 | 1 bits; threshold report mode disabled |  |
| PWR_PERIOD_ENABLE | 0 | 1 bits; periodic report mode disabled |  |
| PWR_REP_DELAY | 0 or 14 | 5 bits; 0 or 56 frames |  |
| RESCAN | 0 | 1 bit; re-initialize and re-acquire the system |  |
| T_ADD | 28 (-14 dB Ec/Io) | 6bits; Pilot detection threshold (-14 dB Ec/Io) |  |
| T_DROP | 32 | 6 bits; Pilot drop threshold (-16 dB Ec/Io) |  |
| T_COMP | 5 | 4; Active set versus candidate set comparison threshold (2.5 dB) |  |
| T_TDROP | 3 | 4 bits; Drop timer value (4 sec) |  |
| EXT_SYS_PARAMETER | 1 | 1 bit; Extended System Parameters message present |  |
| EXT_NGHBR_LST | 1 | 1 bit; Extended neighbor list not present | BandClass1_3_4 |
|  | 0 |  | Not BandClass1_3_4 |
| GEN_NGHBR_LST | 0 | 1 bit; General neighbor list not sent |  |
| GLOBAL_REDIRECT | 0 | 1 bit; Global service redirection message not transmitted |  |
| PRI_NGHBR_LST | 0 | 1 bit; Private neighbor list message not transmitted |  |
| USER_ZONE_ID | 0 | 1 bit; User zone Identification not transmitted |  |
| EXT_GLOBAL_REDIRECT | 0 | 1 bit; Extended global redirection not transmitted |  |
| EXT_CHAN_LST | 0 | 1 bit; Extended channel list not sent |  |

| Condition | Explanation |
| --- | --- |
| BandClass1_4 | If Band Class is 1 or 4 |
| BandClass1_3_4 | If Band Class is 1 or 3 or 4 |

-	Access Parameters Message

Table 4.4.7.2.2-3: Access Parameters message

| Derivation Path: C.S0005 clause 3.7.2.3.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PILOT_PN | See table 4.4.2-4 | 9 bits; |  |
| ACC_MSG_SEQ | 0,1,2..63 | 6 bits; Access Message Sequence; 0 is used in initial transmission and incremented by 1 modulo 64 every time Access Parameters message is modified; for simplicity reasons any value may be used |  |
| ACC_CHAN | 0 | 5 bits; Number of access channels -1 |  |
| NOM_PWR | 0 | 4 bits; Nominal transmit power offset in dB |  |
| INIT_PWR | 0 | 5 bits; Initial power offset in dB |  |
| PWR_STEP | 1 or 3 | 3 bits; Power increment |  |
| NUM_STEP | Any valid value | 4 bits; 5 access probes/ sequence |  |
| MAX_CAP_SZ | 2 or 7 | 3 bits; Maximum Access channel message capsule size =2 or 7 +3 frames |  |
| PAM_SZ | Any valid value | 4 bits; Access Channel preamble length = 2+1 frames |  |
| PSIST(0-9) | 0 | 6 bits; Persistence value for access overload classes 0-9 |  |
| PSIST(10) | 0 | 3 bits; Persistence value for access overload classes 10 |  |
| PSIST(11) | 0 | 3 bits; Persistence value for access overload classes 11 (emergency) |  |
| PSIST(12) | 0 | 3 bits; Persistence value for access overload classes 12 |  |
| PSIST(13) | 0 | 3 bits; Persistence value for access overload classes 13 |  |
| PSIST(14) | 0 | 3 bits; Persistence value for access overload classes 14 |  |
| PSIST(15) | 0 | 3 bits; Persistence value for access overload classes 15 |  |
| MSG_PSIST | 0 | 3 bits; Persistence modifier for access channel attempts for message transmissions |  |
| REG_PSIST | 0 | 3 bits; Persistence modifier for Access Channel attempts for registrations |  |
| PROBE_PN_RAN | 4 or 0 | 4 bits; Time Randomization for Access Channel probes= 0 to 15 PN chips |  |
| ACC_TMO | 3, 4 or 5 | 4 bits; Acknowledgement timeout = (value +2) * 80 ms |  |
| PROBE_BKOFF | 1 or 0 | 4 bits; Access Channel probe back off = no back off |  |
| BKOFF | 1 or 0 | 4 bits; Access channel probe sequence backoff range =no back off |  |
| MAX_REQ_SEQ | Any valid value | 4 bits; Max number of access probe sequences for an access channel request |  |
| MAX_RSP_SEQ | Any valid value | 4 bits; Max number of access probe sequences for an access channel response |  |
| AUTH | 0 | 2 bits; No authentication data in access channel messages |  |
| RAND | Not present | 0 or 32 bits |  |
| NOM_PWR_EXT | 0 | 1 bits; Extended nominal transmit power |  |

-	Extended System Parameters Message

Table 4.4.7.2.2-4: Extended System Parameters Message

| Derivation Path: C.S0005 clause 3.7.2.3.2.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PILOT_PN | See table 4.4.2-4 | 9 bits |  |
| CONFIG_MSG_SEQ | Set to ' CONFIG_SEQ' of base station as per clause 4.4.7.2.1 | 6 bits |  |
| DELETE_FOR_TMSI | 0 | 1 bits; Delete foreign TMSI |  |
| USE_TMSI | 0 or 1 | 1 bits; Use TMSI indicator |  |
| PREF_MSID_TYPE | 3 | 2 bits; Preferred Access Channel MSID Type |  |
| MCC | See table 4.4.2-4 | 10 bits |  |
| IMSI_11_12 | See table 4.4.2-4 | 7 bits; 11th and 12th digits of the IMSI |  |
| TMSI_ZONE_LEN | 1 or 0 | 4 bits |  |
| TMSI_ZONE | 1 or Not present | 8 bits if present |  |
| BCAST_INDEX | 0 | 3 bits |  |
| IMSI_T_SUPPORTED | 0 | 1 bits |  |
| P_REV | 6 | 8 bits |  |
| MIN_P_REV | 6 | 8 bits |  |
| SOFT_SLOPE | 0 | 6 bits |  |
| ADD_INTERCEPT | 0 | 6 bits; 0 dB |  |
| DROP_INTERCEPT | 0 | 6 bits; 0 dB |  |
| PACKET_ZONE_ID | 0 | 8 bits; Packet data service zone not supported |  |
| MAX_NUM_ALT_SO | 0 | 3 bits |  |
| RESELECT_INCLUDED | 0 | 1 bits; System reselection parameters not included |  |
| EC_THRESH | Not present | 0 or 5 bits |  |
| EC_I0_THRESH | Not present | 0 or 5 bits |  |
| PILOT_REPORT | 1 or 0 | 1 bits; Pilot Report indicator; the MS reports or does not report additional pilots which have strengths exceeding T_ADD |  |
| NGHBR_SET_ENTRY_INFO | 0 | 1 bits |  |
| ACC_ENT_HO_ORDER | Not present | 0 or 1 bits |  |
| NGHBR_SET_ACCESS_INFO | 0 | 1 bits |  |
| ACCESS_HO | Not present | 0 or 1 bits |  |
| ACCESS_HO_MSG_RSP | Not present | 0 or 1 bits |  |
| ACCESS_PROBE_HO | Not present | 0 or 1 bits |  |
| ACC_HO_LIST_UPD | Not present | 0 or 1 bits |  |
| ACC_PROBE_HO_OTHER_MSG | Not present | 0 or 1 bits |  |
| MAX_NUM_PROBE_HO | Not present | 0 or 3 bits |  |
| NGHBR_SET_SIZE | Not present | 0 or 6 bits |  |
| ACCESS_ENTRY_HO | Not present | 1 bits; Access Entry Handoff not permitted |  |
| ACCESS_HO_ALLOWED | Not present | 1 bits |  |
| BROADCAST_GPS_ASST | 0 | 1 bits; Broad cast GPS not supported |  |
| QPCH_SUPPORTED | 0 | 1 bits; Quick PCH not supported |  |
| NUM_QPCH | Not present | 0 or 2 bits |  |
| QPCH_RATE | Not present | 0 or 1 bits |  |
| QPCH_POWER_LEVEL_PAGE | Not present | 0 or 3 bits |  |
| QPCH_CCI_SUPPORTED | Not present | 0 or 1 bits |  |
| QPCH_POWER_LEVEL_CONFIG | Not present | 0 or 3 bits |  |
| SDB_SUPPORTED | 0 | 1 bits; Short Data burst not supported |  |
| RLGAIN_TRAFFIC_PILOT | 0 | 6 bits; 0 dB Gain adjustment of Reverse traffic channel |  |
| REV_PWR_CNTL_DELAY_INCL | 0 | 1 bits |  |
| REV_PWR_CNTL_DELAY | Not present | 0 or 2 bits |  |

-	Neighbor List Message

This message is sent when EXT_NGHBR_LST in Systems parameter message is set as '0'B i.e. the band class is 0;

Table 4.4.7.2.2-6: Neighbor List Message

| Derivation Path: C.S0005 clause 3.7.2.3.2.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PILOT_PN | See table 4.4.2-4 | 9 bits |  |
| CONFIG_MSG_SEQ | Set to ' CONFIG_SEQ' of base station as per clause 4.4.7.2.1 | 6 bits |  |
| PILOT_INC | 10 or 1 | 4 bits; Pilot PN Sequence offset index increment = n * 50 |  |
| NGHBR_CONFIG(n) | 0 or Not present |  |  |
| NGHBR_PN(n) | See table 4.4.2-4 or Not Present | 9 bits if present |  |
| Note 1: the elements with index n are repeated as per table 4.4.2-5 |  |  |  |

-	Extended Neighbor List Message

This message is sent when EXT_NGHBR_LST in Systems parameter message is set as '1'B i.e. the band class is not 0;

Table 4.4.7.2.2-7: Extended Neighbor List Message

| Derivation Path: C.S0005 clause 3.7.2.3.2.14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PILOT_PN | See table 4.4.2-4 | 9 bits |  |
| CONFIG_MSG_SEQ | Set to ' CONFIG_SEQ' of base station as per clause 4.4.7.2.1 | 6 bits |  |
| PILOT_INC | 10 or 1 | 4 bits; Pilot PN Sequence offset index increment = n * 50 |  |
| NGHBR_CONFIG(n) | 0 or Not present | 3 bits if present; Neighbor configuration where neighbor base station has same number of frequencies having paging channels as the current base station; |  |
| NGHBR_PN(n) | See table 4.4.2-4 or Not present | 9 bits if present |  |
| SEARCH_PRIORITY(n) | 1, 2 or Not present | 2 bits if present; Medium search priority |  |
| FREQ_INCL(n) | 1 | 1 bit | InterFreqCell |
|  | 0 or Not present |  |  |
| NGHBR_BAND(n) | Set according to PIXIT parameter | 0 or 5 bits | InterFreqCell |
|  | Not present |  |  |
| NGHBR_FREQ(n) | Set according to PIXIT parameter | 0 or 11 bits | InterFreqCell |
|  | Not Present |  |  |
| Note 1: The elements with index n are repeated as per table 4.4.2-5. The SS can transmit up to 7 neighbour intra frequency cell information. The parameters for the first cell should be set as per table 4.4.2-4. |  |  |  |

| Condition | Explanation |
| --- | --- |
| InterFreqCell | If the configured neighbour cell is on a different frequency compared to the cell in which Extended Neighbour list message is being transmitted |

-	Sync channel Messages

Table 4.4.7.2.2-8: Sync Channel Message

| Derivation Path: C.S0005 clause 3.7.2.3.2.26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P_REV | 6 | 8 bits; Protocol revision level |  |
| MIN_P_REV | 6 | 8 bits; Minimum protocol revision level |  |
| SID | See table 4.4.2-4 | 15 bits |  |
| NID | See table 4.4.2-4 | 16 bits |  |
| PILOT_PN | See table 4.4.2-4 | 9 bits |  |
| LC_STATE | A valid value, calculated and updated by the SS | 42 bits; Long code State |  |
| SYS_TIME | A valid value, calculated and updated by the SS | 36 bits; System time |  |
| LP_SEC | A valid value, calculated and updated by the SS | 8 bits; The number of leap seconds |  |
| LTM_OFF | A valid value, calculated and updated by the SS | 6 bits; Offset of local time from System time |  |
| DAYLT | A valid value, calculated and updated by the SS | 1 bits; Daylight savings indicator |  |
| PRAT | 0 | 2 bits; Paging Channel Data rate=9600 bps |  |
| CDMA_FREQ | Set according to PIXIT parameter or as per clause 4.3.1.4 | 11 bits; Frequency assignment |  |
| EXT_CDMA_FREQ | Set according to PIXIT parameter or as per clause 4.3.1.4 | 11 bits; Extended frequency assignment |  |

### 4.4.8	Common parameters for simulated WLAN AP’s

The parameters specified in this subclause apply to all simulated WLAN APs unless otherwise specified.

Default WLAN parameters for simulated APs are specified in table 4.4.8-1.

Table 4.4.8-1: Default parameters for simulated WLAN APs

| Parameter | Value | Condition |
| --- | --- | --- |
| SSID | 3GPP WLAN InterWorking |  |
| Security | [WPA2] |  |
| Hot Spot 2.0 Support | Yes |  |
| Trusted/Un-Trusted non 3GPP Access | Un-Trusted |  |
| EAP-AKA Procedure | CHAP |  |
| Beacon RSSI | 42 (-86 dBm), NOTE 1 | OFFLOAD |
| Beacon RSSI | Configured at the SS according to PIXIT parameter | IMSoWLAN |
| NOTE 1: The power level is specified at each UE Rx antenna. |  |  |

| Condition | Explanation |
| --- | --- |
| OFFLOAD | WLAN offload scenarios |
| IMSoWLAN | IMS over WLAN scenarios |

Table 4.4.8-2: Default parameters for simulated WLAN APs (2nd WLAN)

| Parameter | Value | Condition |
| --- | --- | --- |
| SSID | 3GPP WLAN InterWorking 2 |  |
| Security | [WPA2] |  |
| Hot Spot 2.0 Support | Yes |  |
| Trusted/Un-Trusted non 3GPP Access | Un-Trusted |  |
| EAP-AKA Procedure | CHAP |  |
| Beacon RSSI | 42 (-86 dBm), NOTE 1 | OFFLOAD |
| Beacon RSSI | Configured at the SS according to PIXIT parameter | IMSoWLAN |
| NOTE 1: The power level is specified at each UE Rx antenna. |  |  |

| Condition | Explanation |
| --- | --- |
| OFFLOAD | WLAN offload scenarios |
| IMSoWLAN | IMS over WLAN scenarios |

### 4.4.9	Common parameters for simulated Bluetooth beacon’s

The parameters specified in this subclause apply to all simulated Bluetooth beacons unless otherwise specified.

Default Bluetooth parameters for simulated beacons are specified in table 4.4.9-1.

Table 4.4.9-1: Default parameters for simulated Bluetooth beacons

| Parameter | Value | Condition |
| --- | --- | --- |
| Bluetooth Core Specification | 4.0 and forward |  |
| Classic Bluetooth/ BLE | Classic Bluetooth |  |
| LOCAL NAME | 3GPP BT InterWorking |  |
| Beacon RSSI | 42 (-86 dBm), NOTE 1 |  |
| NOTE 1: The power level is specified at each UE Rx antenna. |  |  |

Table 4.4.9-2: Default parameters for simulated Bluetooth beacons (2nd Bluetooth)

| Parameter | Value | Condition |
| --- | --- | --- |
| Bluetooth Core Specification | 4.0 and forward |  |
| Classic Bluetooth/ BLE | Classic Bluetooth |  |
| LOCAL NAME | 3GPP BT InterWorking 2 |  |
| Beacon RSSI | 42 (-86 dBm), NOTE 1 |  |
| NOTE 1: The power level is specified at each UE Rx antenna. |  |  |

## 4.5	Generic procedures

This clause describes UE test states which can be used in the initial condition of many test cases defined in TS 36.521-1 [21], TS 36.523-1 [18] and TS 36.523-3 [34] or other procedures defined in this specification. This section also defines a set of procedures to bring the UE into these states.

### 4.5.1	UE test states

NOTE:	The need to have a procedure for the transition from State 4/State 3 to State 2 is for further study and it can added if the technical motivation for this procedure can be justified.

Figure 4.5.1-1: E-UTRAN UE Test States for Basic Generic Procedures

Figure 4.5.1-2: E-UTRAN UE Test States for CIoT

NOTE:	Transition to state 5 is performed only when Test Loop Mode E is required to be performed when UE is out of coverage.

In order that the UE can set up a call or session in E-UTRAN, there are a number of procedures to be undertaken in a hierachical sequence to move between known states. The sequences are shown in figure 4.5.1-1 and the status of the relevant protocols in the UE in the different states are given in table 4.5.1-1.

Table 4.5.1-1: The E-UTRAN UE states

|  |  | RRC | ECM | EMM | ESM | UE Test Mode |
| --- | --- | --- | --- | --- | --- | --- |
| State 1 | Switched OFF | ----- | ----- | ----- | ----- | ----- |
| State 2 | Registered, Idle Mode | RRC_IDLE | ECM-IDLE | EMM-REGISTERED | 1 default EPS bearer context active. 2 default EPS bearer contexts active (only for RAN Assisted WLAN interworking) | Not active |
| State 2-CE | Registered, Idle Mode, Cell supporting BL/CE UE | RRC_IDLE | ECM-IDLE | EMM-REGISTERED | 1 default EPS bearer context active. | Not active |
| State 2A | Registered, Idle Mode, UE Test Mode Activated | RRC_IDLE | ECM-IDLE | EMM-REGISTERED | 1 default EPS bearer context active. | Active |
| State 2A-CE | Registered, Idle Mode, Cell supporting BL/CE UE, UE Test Mode Activated | RRC_IDLE | ECM-IDLE | EMM-REGISTERED | 1 default EPS bearer context active. | Active |
| State 2B | Registered, Idle Mode, pre-registered on HRPD | RRC_IDLE | ECM-IDLE | EMM-REGISTERED | 1 default EPS bearer context active. | Active |
| State 2C | Registered, Idle Mode, pre-registered on 1xRTT | RRC_IDLE | ECM-IDLE | EMM-REGISTERED | 1 default EPS bearer context active. | Active |
| State 3 | Generic RB Established | RRC_CONNECTED 1 + N (0 ≤ N ≤ 7) data radio bearers configured as specified in the test cases. 2 + N (0 ≤ N ≤ 7) data radio bearers configured as specified in the test cases (only for for RAN Assisted WLAN interworking) | ECM-CONNECTED | EMM-REGISTERED | 1 default EPS bearer context active and N (0 ≤ N ≤ 7) dedicated EPS bearers active as specified in the test cases. 2 default EPS bearer contexts active and N (0 ≤ N ≤ 7) dedicated EPS bearers active as specified in the test cases (only for RAN Assisted WLAN interworking) | Not active |
| State 3-CP | CIoT CP Connection Established | RRC_CONNECTED | ECM-CONNECTED | EMM-REGISTERED | - | Not active |
| State 3A-CP | CIoT CP Connection Established, UE Test Mode Activated | RRC_CONNECTED | ECM-CONNECTED | EMM-REGISTERED | - | Active |
| State 3-UP | CIoT UP Connection Established | RRC_CONNECTED | ECM-CONNECTED | EMM-REGISTERED | 1 default EPS bearer context active. | Not active |
| State 3A-UP | CIoT UP Connection Established, UE Test Mode Activated | RRC_CONNECTED | ECM-CONNECTED | EMM-REGISTERED | 1 default EPS bearer context active. | Active |
| State 4A-UP | Loopback Activation User Plane | RRC_CONNECTED | ECM-CONNECTED | EMM-REGISTERED | 1 default EPS bearer context active. | Active |
| State 3-CE | Generic RB Established, Cell supporting BL/CE UE | RRC_CONNECTED 1 + N (0 ≤ N ≤ 7) data radio bearers configured as specified in the test cases. | ECM-CONNECTED | EMM-REGISTERED | 1 default EPS bearer context active and N (0 ≤ N ≤ 7) dedicated EPS bearers active as specified in the test cases. | Not active |
| State 3A | Generic RB Established, UE Test Mode Activated | RRC_CONNECTED 1 + N (0 ≤ N ≤ 7) data radio bearers configured as specified in the test cases | ECM-CONNECTED | EMM-REGISTERED | 1 default EPS bearer context active and N (0 ≤ N ≤ 7) dedicated EPS bearers active as specified in the test cases | Active |
| State 3A-CE | Generic RB Established, UE Test Mode Activated, Cell supporting BL/CE UE, UE Test Mode Activated | RRC_CONNECTED 1 + N (0 ≤ N ≤ 7) data radio bearers configured as specified in the test cases | ECM-CONNECTED | EMM-REGISTERED | 1 default EPS bearer context active and N (0 ≤ N ≤ 7) dedicated EPS bearers active as specified in the test cases | Active |
| State 3B | Generic RB Established, pre-registered on HRPD | RRC_CONNECTED 1 + N (0 ≤ N ≤ 7) data radio bearers configured as specified in the test cases | ECM-CONNECTED | EMM-REGISTERED | 1 default EPS bearer context active and N (0 ≤ N ≤ 7) dedicated EPS bearers active as specified in the test cases | Active |
| State 3C | Generic RB Established, pre-registered on 1xRTT | RRC_CONNECTED 1 + N (0 ≤ N ≤ 7) data radio bearers configured as specified in the test cases | ECM-CONNECTED | EMM-REGISTERED | 1 default EPS bearer context active and N (0 ≤ N ≤ 7) dedicated EPS bearers active as specified in the test cases | Active |
| State 4 | Loopback Activated | RRC_CONNECTED 1 + N (0 ≤ N ≤ 7) data radio bearers configured as specified in the test cases | ECM-CONNECTED | EMM-REGISTERED | 1 default EPS bearer context active and N (0 ≤ N ≤ 7) dedicated EPS bearers active as specified in the test cases | Active |
| State 4-CE | Loopback Activated, Cell supporting BL/CE UE | RRC_CONNECTED 1 data radio bearer configured | ECM-CONNECTED | EMM-REGISTERED | 1 default EPS bearer context active | Active |
| State 5 | Out of Coverage | Any Cell Selection state | ----- | ----- | ----- | Active |
| State 5-V2X | Out of Coverage, V2X setup | Any Cell Selection state | ----- | ----- | ----- | Not active |
| State 5A-V2X | Out of Coverage, Loopback Activated, V2X setup | Any Cell Selection state | ----- | ----- | ----- | Active |

NOTE: Refer to TS 24.301 [28] subclause 5.5.1.1 for more details on the ESM state.

NOTE: Refer to TS 36.509 [38] for details regarding UE test mode and UE Loopback.

Figure 4.5.1-2: HRPD UE Test States for Basic Generic Procedures

In order for the UE to set up a session in an HRPD RAN, there are a number of procedures that need to be performed in a certain sequence to move between known UE states. These states and sequences are shown in figure 4.5.1-2. HRPD UE states are described in table 4.5.1-2.

Table 4.5.1-2: Description of HRPD UE states

| State | Description |
| --- | --- |
| H2 | The UE is registered on the HRPD network, but has no active connection to the network. The Inter-RAT Idle State Protocol (IR-ISP) is in Monitor State. The UE should be ready to perform a non-optimised cell reselection to E-UTRAN (as per [41] cl. 8 and [42] cl. 14.1). |
| H3 | The UE is registered on the HRPD network, and has an active network connection. The UE should be ready to perform a non-optimised handover to E-UTRAN (as per [41] cl. 8). |
| H2A | Same state as State H2, but the UE is also pre-registered in E-UTRAN. The UE should be ready to perform an optimised cell reselection to E-UTRAN (as per [41] cl. 9.6). |
| H3A | Same state as State H3, but the UE is also pre-registered in E-UTRAN. The UE should be ready to perform an optimised handover to E-UTRAN (as per [41] cl. 9.5). |

### 4.5.2	UE Registration (State 2)

4.5.2.0	General

IF pc_ue_CategoryDL_M1 THEN use the procedure for State 2-CE according to clause 4.5.2AA ELSE use the procedure defined in the present clause.

NOTE:	The statement to use alternate procedure for CAT M1 UE (pc_ue_CategoryDL_M1 is set) is to enable LTE legacy protocol test cases to be used for CAT M1 UE testing. Any CAT M1 or CE specific test cases should not rely on this statement, but refer directly to the relevant CE specific state.

UE Registration depends on the specific configuration of a UE, which is expressed by PICS according to TS 36.523-2 [19]. In order to reduce complexity and to achieve testability several assumptions and definitions are made:

-	An IMS capable UE is compliant to GSMA PRD IR.92 [52];

-	During initial UE registration the UE requests for one or two PDN connectivities;

-	When the UE requests for two PDN connectivities:

-	it re-uses the existing RRC connection for signalling regarding the 2nd PDN connectivity;

-	one of the PDN connectivities is for IMS;

-	If the UE is configured with an APN for XCAP only usage (pc_XCAP_only_APN==true), connectivity to this PDN is not established during the initial UE registration;

-	As for any 2nd PDN connectivity request security protection is already established the UE provides a valid APN in the PDN CONNECTIVITY REQUEST.

In detail the following UE configurations are supported:

| Configuration | Condition | Explanation |
| --- | --- | --- |
| IMS_Only | pc_IMS AND NOT pc_Provide_Internet_as_second_APN AND NOT pc_Provide_IMS_as_second_APN | UE is configured to request for IMS PDN connectivity only |
| Internet_Only | NOT pc_IMS | UE does not support IMS |
| IMS_Internet | pc_IMS AND pc_Provide_Internet_as_second_APN | UE is configured to request for IMS PDN connectivity first and then to request for an internet PDN connectivity |
| Internet_IMS | pc_IMS AND pc_Provide_IMS_as_second_APN | UE is configured to request for an internet PDN connectivity first and then to request for the IMS PDN connectivity |
| NOTE 1: pc_Provide_Internet_as_second_APN and pc_Provide_IMS_as_second_APN are mutually exclusive i.e. shall not be true at the same time. NOTE 2: IMS_Only configuration is not considered for EN-DC test cases defined in TS 38.523-1 [71] |  |  |

Further auxiliary definitions:

| Configuration | Condition |
| --- | --- |
| PDN1_IMS | IMS_Only OR IMS_Internet |
| PDN2_IMS | Internet_IMS |
| PDN1_INTERNET_ONLY | Internet_Only |
| MULTI_PDN | IMS_Internet OR Internet_IMS |
| XCAP_SIGNALLING | test case requires XCAP signalling |
| EN-DC | Applicable to EN-DC test cases defined in TS 38.523-1 [71] |
| NR | Applicable to NR test cases defined in TS 38.523-1 [71] |

4.5.2.1	Initial conditions

System Simulator:

-	1 cell, default parameters.

-	The procedure shall be performed under ideal radio conditions as defined in clause 5

User Equipment:

-	The Test UICC shall be inserted. This shall contain either ISIM and USIM applications or only a USIM application on UICC.

-	If RAN Assisted WLAN interworking test cases are being performed for ANDSF rules using RAN provided thresholds, ANDSF rules to match the RAN rules defined in TS 36.304 shall be available in the UE.

4.5.2.2	Definition of system information messages

The default system information messages are used.

4.5.2.3	Procedure

Table 4.5.2.3-1: UE registration procedure (state 1 to state 2)

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 |  | <-- | RRC: SYSTEM INFORMATION (BCCH) |
| 2 | UE transmits an RRCConnectionRequest message. | --> | RRC: RRCConnectionRequest |
| 3 | SS transmits an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| 4 | The UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the Attach procedure by including the ATTACH REQUEST message. The PDN CONNECTIVITY REQUEST message is piggybacked in ATTACH REQUEST | --> | RRC: RRCConnectionSetupComplete NAS: ATTACH REQUEST NAS: PDN CONNECTIVITY REQUEST |
| 5 | The SS transmits an AUTHENTICATION REQUEST message to initiate the EPS authentication and AKA procedure. | <-- | RRC: DLInformationTransfer NAS: AUTHENTICATION REQUEST |
| 6 | The UE transmits an AUTHENTICATION RESPONSE message and establishes mutual authentication. | --> | RRC: ULInformationTransfer NAS: AUTHENTICATION RESPONSE |
| 7 | The SS transmits a NAS SECURITY MODE COMMAND message to activate NAS security. | <-- | RRC: DLInformationTransfer NAS: SECURITY MODE COMMAND |
| 8 | The UE transmits a NAS SECURITY MODE COMPLETE message and establishes the initial security configuration. | --> | RRC: ULInformationTransfer NAS: SECURITY MODE COMPLETE |
| - | EXCEPTION: Steps 9a1 to 9a2 describe behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that take place if the UE has ESM information which needs to be transferred. | - | - |
| 9a1 | IF the UE sets the ESM information transfer flag in the last PDN CONNECTIVITY REQUEST message THEN the SS transmits an ESM INFORMATION REQUEST message to initiate exchange of protocol configuration options and/or APN. | <-- | RRC: DLInformationTransfer NAS: ESM INFORMATION REQUEST |
| 9a2 | The UE transmits an ESM INFORMATION RESPONSE message to transfer protocol configuration options and/or APN. | --> | RRC: ULInformationTransfer NAS: ESM INFORMATION RESPONSE |
| 10 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |
| 11 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 12 | The SS transmits a UECapabilityEnquiry message including RAT-Type eutra to initiate the UE radio access capability transfer procedure. | <-- | RRC: UECapabilityEnquiry |
| 13 | The UE transmits a UECapabilityInformation message to transfer UE radio access capability. | --> | RRC: UECapabilityInformation |
| - | EXCEPTION: Steps 13Aa1 to 13Aa2 describe behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place depending on the UE implementation. | - | - |
| 13Aa1 | IF EN-DC OR NR THEN, the SS transmits a UECapabilityEnquiry message to initiate the UE radio access capability transfer procedure. | <-- | RRC: UECapabilityEnquiry |
| 13Aa2 | The UE transmits a UECapabilityInformation message to transfer UE radio access capability. | --> | RRC: UECapabilityInformation |
| 14 | The SS transmits an RRCConnectionReconfiguration message to establish the default bearer with condition SRB2-DRB(1, 0) according to 4.8.2.2.1.1. This message includes the ATTACH ACCEPT message. The ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message is piggybacked in ATTACH ACCEPT. | <-- | RRC: RRCConnectionReconfiguration NAS: ATTACH ACCEPT NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST |
| 15 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of default bearer. | --> | RRC: RRCConnectionReconfigurationComplete |
| - | EXCEPTION: In parallel to the event described in step 16 below, if initiated by the UE the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane. | - | - |
| - | EXCEPTION: IF PDN1_IMS THEN in parallel to the event described in step 16 below the generic procedure for IMS signalling in the U-plane specified in subclause 4.5A.3 takes place if requested by the UE | - | - |
| 16 | This message includes the ATTACH COMPLETE message. The ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message is piggybacked in ATTACH COMPLETE. | --> | RRC: ULInformationTransfer NAS: ATTACH COMPLETE NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |
| - | EXCEPTION: Steps 16a1 to 16c1 describe behaviour that depends on test case requirements and UE behaviour; the "lower case letter" identifies the different branches. | - | - |
| 16a1 | IF IMS VoPS is set to ‘0’ in ATTACH ACCEPT at step 14 THEN the procedure according to Table 4.5.2.3-2 takes place | - | - |
| 16a2-16b6 | Void. | - | - |
| 16c1 | ELSE IF MULTI_PDN = TRUE THEN The generic procedure for UE establishing additional PDN connectivity as specified subclause 4.5A.16 takes place | - | - |
| 17 | The SS transmits an RRCConnectionRelease message to release RRC connection and move to RRC_IDLE (State 2). | <-- | RRC: RRCConnectionRelease |
| - | EXCEPTION: Steps18a1 to 18b1 describe behaviour that depends on UE capabilities/configuration and test requirements; the "lower case letter" identifies a step sequence that takes place if one of those is supported/configured. | - | - |
| 18a1 | IF additional PDN has been established AND (pc_XCAP_only_APN OR NOT XCAP_SIGNALLING) AND pc_UE_supports_user_initiated_PDN_disconnect THEN the non-IMS PDN shall be released as specified in TS 36.508 [2]. subclause 4.5A.17 | - | - |
| 18b1 | IF additional PDN has been established AND (pc_XCAP_only_APN OR NOT XCAP_SIGNALLING) AND NOT pc_UE_supports_user_initiated_PDN_disconnect THEN the non-IMS PDN shall be released as specified in subclause 4.5A.18 | - | - |
| NOTE 1: Void NOTE 2: Void |  |  |  |

Table 4.5.2.3-2: Specific behaviour when IMS voice is not supported by the network

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: Steps 1a1 to 1b6 describe behaviour that depends on UE configuration; the "lower case letter" identifies the different branches | - | - |
| 1a1 | IF pc_voice_PS_1_CS_2, pc_attach and pc_TAU_connected_in_IMS are set to TRUE (Note 1) THEN The UE transmits a TRACKING AREA UPDATE REQUEST message. | --> | RRC: ULInformationTransfer NAS: TRACKING AREA UPDATE REQUEST |
| 1a2 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | RRC: DLInformationTransfer NAS: TRACKING AREA UPDATE ACCEPT |
| 1a3 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | RRC: ULInformationTransfer NAS: TRACKING AREA UPDATE COMPLETE |
| 1b1 | ELSE IF pc_voice_PS_1_CS_2, pc_attach and pc_TAU_idle_in_IMS are set to TRUE (Note 2) THEN The SS transmits an RRCConnectionRelease message to release the RRC connection. | <-- | RRC: RRCConnectionRelease |
| 1b2 | The UE transmits an RRCConnectionRequest message. | --> | RRC: RRCConnectionRequest |
| 1b3 | The SS transmit an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| 1b4 | The UE transmits an RRCConnectionSetupComplete message including a TRACKING AREA UPDATE REQUEST message. | --> | RRC: RRCConnectionSetupComplete NAS: TRACKING AREA UPDATE REQUEST |
| 1b5 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | RRC: DLInformationTransfer NAS: TRACKING AREA UPDATE ACCEPT |
| 1b6 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | RRC: ULInformationTransfer NAS: TRACKING AREA UPDATE COMPLETE |
| 2 | Start Timer_1 = 5 s | - | - |
| - | EXCEPTION: Steps 3a1 to 3b1 describe behaviour that depends on UE behaviour; the "lower case letter" identifies the different branches | - | - |
| 3a1 | The generic procedure for UE establishing additional PDN connectivity as specified in TS 36.508 subclause 4.5A.16 takes place | - | - |
| 3a2 | Stop Timer_1 | - | - |
| 3b1 | Timer_1 expires | - | - |
| NOTE 1: The procedure is used with specific message with no IMS voice network support. The UE is configured for voice domain preference IMS PS voice preferred, CS Voice as secondary and to initiate EPS attach. The UE implementation supports TAU in connected mode. NOTE 2: The procedure is used with specific message with no IMS voice network support. The UE is configured for voice domain preference IMS PS voice preferred, CS Voice as secondary and to initiate EPS attach. The UE implementation supports TAU in idle mode. |  |  |  |

4.5.2.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7 with the exceptions below.

Table 4.5.2.4-1: RRCConnectionRequest (Step 2)

| Derivation Path: Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionRequest ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcConnectionRequest-r8 SEQUENCE { |  |  |  |  |
| ue-Identity |  | Any allowed value |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 4.5.2.4-2: UECapabilityInformation (Steps 13 and 13Aa2)

| Derivation Path: Table 4.6.1-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UECapabilityInformation ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| ueCapabilityInformation-r8 |  | Any allowed value |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 4.5.2.4-3: ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST (Step 14)

| Derivation Path: Table 4.7.3-6 with condition IMS_PDN_ConnEstab for PDN1_IMS and NOT IMS_PDN_ConnEstab |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| EPS bearer identity |  | '0101'B | arbitrary value used for PDN connectivity being maintained during the test case | PDN1_IMS OR (PDN2_IMS AND ImsVoPS0_NoImsPDN) OR NOT MULTI_PDN |
| EPS bearer identity |  | ‘1100’B | arbitrary value used for additional PDN connectivity which may be released before entering the test procedure of the test case | MULTI_PDN AND NOT PDN1_IMS AND NOT (PDN2_IMS AND ImsVoPS0_NoImsPDN) |
| EPS bearer identity |  | ‘0101’B | arbitrary value used for IMS PDN connectivity being maintained during EN-DC test case | EN-DC AND PDN1_IMS |
| EPS bearer identity |  | ‘1100’B | arbitrary value used for non-IMS PDN connectivity being maintained during EN-DC test case | EN-DC AND (PDN1_INTERNET_ONLY OR PDN2_IMS) |
| PDN address |  |  |  | IPv4_address_only |
| Length of PDN address contents |  | 5 octets |  |  |
| PDN type value |  | ‘001’B | IPv4 |  |
| PDN address information |  | IPv4 address | The SS provides a valid IPv4 address | NOT IPv4-DHCP |
|  |  | 0.0.0.0 | DHCPv4 is to be used to allocate the IPv4 address | IPv4-DHCP |
| ESM cause |  | IF “PDN type” IE in step 4 is ‘IPv4v6’ THEN ‘00110010’B ELSE Not present | “PDN type IPv4 only allowed” |  |

NOTE:	The default message contents specified in table 4.7.3-6 apply unless the condition IPv4_address_only in table 4.5.2.4-3 is true.

| Condition | Explanation |
| --- | --- |
| IPv4_address_only | This condition applies if the test case preamble description indicates that the UE is allocated an IPv4 address only. |
| IPv4-DHCP | If in the last PDN CONNECTIVITY REQUEST or ESM INFORMATION RESPONSE sent prior to this message, the IE Protocol configuration options contains a configuration protocol option = ‘000B00H’ (“IPv4 address allocation via DHCPv4”, length of contents = 0). Note 1: This condition is used in conjunction with IPv4 or IPv4v6 as indicated in the “PDN address row” just above. Note 2: If both messages, PDN CONNECTIVITY REQUEST and ESM INFORMATION RESPONSE, are received and contain a Protocol configuration options IE then the IE from the message which is received later shall be used. |
| ImsVoPS0_NoImsPDN | If the test case sets IMS VoPS to 0 AND (pc_UE_NoReqIMS_IMSVoPS_0 == TRUE) |

Table 4.5.2.4-4: TRACKING AREA UPDATE REQUEST (Steps 16a1 and 16b4)

Derivation Path: Table 4.7.2-27 with condition combined_TA_LA

Table 4.5.2.4-5: TRACKING AREA UPDATE ACCEPT (Steps 16a2 and 16b5)

Derivation Path: Table 4.7.2-24 with condition combined_TA_LA

### 4.5.2A	UE Registration, UE Test Mode Activated (State 2A)

4.5.2A.0	General

IF pc_ue_CategoryDL_M1 THEN use the procedure for State 2A-CE according to clause 4.5.2AB ELSE use the procedure defined in the present clause.

NOTE:	The statement to use alternate procedure for CAT M1 UE (pc_ue_CategoryDL_M1 is set) is to enable LTE legacy protocol test cases to be used for CAT M1 UE testing. Any CAT M1 or CE specific test cases should not rely on this statement, but refer directly to the relevant CE specific state.

The same assumptions and definitions apply as in clause 4.5.2.

#### 4.5.2A.1	Initial conditions

System Simulator:

-	1 cell, default parameters.

-	The procedure shall be performed under ideal radio conditions as defined in clause 5

User Equipment:

- 	The Test USIM shall be inserted.

4.5.2A.2	Definition of system information messages

The default system information messages are used.

4.5.2A.3	Procedure

Table 4.5.2A.3-1: UE registration with test mode activation procedure (state 1 to state 2A)

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 to 9a2 | Same procedure for steps 1 to 9a2 as specified in the procedure in clause 4.5.2.3 | - | - |
| 10 | The SS transmits an ACTIVATE TEST MODE message to activate UE radio bearer test mode procedure. | <-- | RRC: DLInformationTransfer TC: ACTIVATE TEST MODE |
| 11 | The UE transmits an ACTIVATE TEST MODE COMPLETE message. | --> | RRC: ULInformationTransfer TC: ACTIVATE TEST MODE COMPLETE |
| 12 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |
| 13 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 14 | The SS transmits a UECapabilityEnquiry message including RAT-Type eutra to initiate the UE radio access capability transfer procedure. | <-- | RRC: UECapabilityEnquiry |
| 15 | The UE transmits a UECapabilityInformation message to transfer UE radio access capability. | --> | RRC: UECapabilityInformation |
| - | EXCEPTION: Steps 15Aa1 to 15Aa2 describe behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place depending on the UE implementation. | - | - |
| 15Aa1 | IF EN-DC OR NR THEN the SS transmits a UECapabilityEnquiry message to initiate the UE radio access capability transfer procedure. | <-- | RRC: UECapabilityEnquiry |
| 15Aa2 | The UE transmits a UECapabilityInformation message to transfer UE radio access capability. | --> | RRC: UECapabilityInformation |
| 16 | The SS transmits an RRCConnectionReconfiguration message to establish the default bearer with condition SRB2-DRB(1, 0) according to 4.8.2.2.1.1. This message includes the ATTACH ACCEPT message. The ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message is piggybacked in ATTACH ACCEPT. | <-- | RRC: RRCConnectionReconfiguration NAS: ATTACH ACCEPT NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST |
| 17 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of default bearer. | --> | RRC: RRCConnectionReconfigurationComplete |
| - | EXCEPTION: In parallel to the event described in step 18 below the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane if requested by the UE. | - | - |
| 18 | This message includes the ATTACH COMPLETE message. The ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message is piggybacked in ATTACH COMPLETE. | --> | RRC: ULInformationTransfer NAS: ATTACH COMPLETE NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |
| - | EXCEPTION: Steps18a1 to 18c1 describe behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that takes place when the UE is configured in a certain way. | - | - |
| 18a1 | IF IMS voice not supported and pc_voice_PS_1_CS_2, pc_attach and pc_TAU_connected_in_IMS are set to TRUE (Note 1) THEN The UE transmits a TRACKING AREA UPDATE REQUEST message. | --> | RRC: ULInformationTransfer NAS: TRACKING AREA UPDATE REQUEST |
| 18a2 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | RRC: DLInformationTransfer NAS: TRACKING AREA UPDATE ACCEPT |
| 18a3 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | RRC: ULInformationTransfer NAS: TRACKING AREA UPDATE COMPLETE |
| 18b1 | ELSE IF IMS voice not supported and pc_voice_PS_1_CS_2, pc_attach and pc_TAU_idle_in_IMS are set to TRUE (Note 2) THEN The SS transmits an RRCConnectionRelease message to release the RRC connection. | <-- | RRC: RRCConnectionRelease |
| 18b2 | The UE transmits an RRCConnectionRequest message. | --> | RRC: RRCConnectionRequest |
| 18b3 | The SS transmit an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| 18b4 | The UE transmits an RRCConnectionSetupComplete message including a TRACKING AREA UPDATE REQUEST message. | --> | RRC: RRCConnectionSetupComplete NAS: TRACKING AREA UPDATE REQUEST |
| 18b5 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | RRC: DLInformationTransfer NAS: TRACKING AREA UPDATE ACCEPT |
| 18b6 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | RRC: ULInformationTransfer NAS: TRACKING AREA UPDATE COMPLETE |
| 18c1-18c3 | ELSE IF MULTI_PDN = TRUE THEN Steps from 1 to 3 from the generic procedure for UE establishing additional PDN connectivity as specified in TS 36.508 subclause 4.5A.16 takes place | - | - |
| - | EXCEPTION: In parallel to the event described in step 18c4 below the generic procedure for IP address allocation in the U-plane specified in subclause 4.5A.1 takes place performing IP address allocation in the U-plane if requested by the UE. | - | - |
| 18c4 | The UE transmits an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message | --> | RRC: ULInformationTransfer NAS:ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |
| 19 | The SS transmits an RRCConnectionRelease message to release RRC connection and move to RRC_IDLE (State 2A). | <-- | RRC: RRCConnectionRelease |
| - | EXCEPTION: Steps 20a1 to 20b1 describe behaviour that depends on UE capabilities/configuration; the "lower case letter" identifies a step sequence that takes place if one of those is supported/configured. | - | - |
| 20a1 | IF MULTI_PDN AND pc_UE_supports_user_initiated_PDN_disconnect THEN the non-IMS PDN shall be released as specified in subclause 4.5A.17 | - | - |
| 20b1 | IF MULTI_PDN AND NOT pc_UE_supports_user_initiated_PDN_disconnect THEN the non-IMS PDN shall be released as specified in subclause 4.5A.18 | - | - |
| NOTE 1: The procedure is used with specific message with no IMS voice network support. The UE is configured for voice domain preference IMS PS voice preferred, CS Voice as secondary and to initiate EPS attach. The UE implementation supports TAU in connected mode. NOTE 2: The procedure is used with specific message with no IMS voice network support. The UE is configured for voice domain preference IMS PS voice preferred, CS Voice as secondary and to initiate EPS attach. The UE implementation supports TAU in idle mode. |  |  |  |

4.5.2A.4	Specific message contents

All specific message contents shall be referred to clause 4.6, 4.7 and 4.7A with the exceptions below.

Table 4.5.2A.4-1: RRCConnectionRequest (Step 2)

| Derivation Path: Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionRequest ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcConnectionRequest-r8 SEQUENCE { |  |  |  |  |
| ue-Identity |  | Any allowed value |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 4.5.2A.4-2: UECapabilityInformation (Steps 15 and 15Aa2)

| Derivation Path: Table 4.6.1-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UECapabilityInformation ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| ueCapabilityInformation-r8 |  | Any allowed value |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 4.5.2A.4-3: TRACKING AREA UPDATE REQUEST (Steps 18a1 and 18b4)

Derivation Path: Table 4.7.2-27 with condition combined_TA_LA

Table 4.5.2A.4-4: TRACKING AREA UPDATE ACCEPT (Steps 18a2 and 18b5)

Derivation Path: Table 4.7.2-24 with condition combined_TA_LA

### 4.5.2AA	UE Registration in cell supporting BL/CE UE (State 2-CE)

The same assumptions and definitions apply as in clause 4.5.2.

#### 4.5.2AA.1	Initial conditions

System Simulator:

-	1 cell, default parameters.

-	The procedure shall be performed under ideal radio conditions as defined in clause 5

User Equipment:

- 	The Test USIM shall be inserted.

#### 4.5.2AA.2	Definition of system information messages

The default system information messages are used with the exceptions below.

Table 4.5.2AA.2-1: MasterInformationBlock

Derivation Path: 36.508 Table 4.4.3.2-1 using condition CEmodeB if the test case specifically tests CE mode B else use condition CEmodeA.

Table 4.5.2AA.2-2: SystemInformation-BR-r13

Derivation Path: 36.508 Table 4.4.3.2-2A.

Table 4.5.2AA.2-3: SystemInformationBlockType1-BR-r13

Derivation Path: 36.508 Table 4.4.3.2-3A using condition ENHANCED COVERAGE if the BL/CE UE initial condition is intended to be in enhanced coverage; else use condition NORMAL COVERAGE.

#### 4.5.2AA.3	Procedure

Table 4.5.2AA.3-1: UE registration procedure (state 1 to state 2-CE)

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | In addition to the system information broadcasted on BCCH the SS broadcasts SystemInformation-BR-r13, SystemInformationBlockType1-BR-r13 and other relevant system information blocks as required by the specified combinations of system information blocks specified for the test case on BCCH-BR. | <-- | RRC: SYSTEM INFORMATION (BCCH and BCCH-BR) |
| 2 to 18b1 | Same procedure for steps 2 to 18b1 as specified in the procedure in clause 4.5.2.3 | - | - |

#### 4.5.2AA.4	Specific message contents

Same specific message content as in clause 4.5.2.4 with the exception below:

-	In addition to any other conditions use the condition CEmodeB if the test case specifically tests CE mode B else use condition CEmodeA.

### 4.5.2AB	UE Registration, UE Test Mode Activated in cell supporting BL/CE UE (State 2A-CE)

The same assumptions and definitions apply as in clause 4.5.2.

#### 4.5.2AB.1	Initial conditions

System Simulator:

-	1 cell, default parameters.

-	The procedure shall be performed under ideal radio conditions as defined in clause 5

User Equipment:

- 	The Test USIM shall be inserted.

#### 4.5.2AB.2	Definition of system information messages

Same system information message content as in clause 4.5.2AA.2.

#### 4.5.2AB.3	Procedure

Table 4.5.2AB.3-1: UE registration procedure (state 1 to state 2A-CE)

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | In addition to the system information broadcasted on BCCH the SS broadcasts SystemInformation-BR-r13, SystemInformationBlockType1-BR-r13 and other relevant system information blocks as required by the specified combinations of system information blocks specified for the test case on BCCH-BR. | <-- | RRC: SYSTEM INFORMATION (BCCH and BCCH-BR) |
| 2 to 9a2 | Same procedure for steps 2 to 9a2 as specified in the procedure in clause 4.5.2.3 | - | - |
| 10 to 20b1 | Same procedure for steps 10 to 20b1 as specified in the procedure in clause 4.5.2A.3 | - | - |

#### 4.5.2AB.4	Specific message contents

Same specific message content as in clause 4.5.2A.4 with the exception below:

-	In addition to any other conditions use the condition CEmodeB if the test case specifically tests CE mode B else use condition CEmodeA.

### 4.5.2B	UE Registration, pre-registration on HRPD (State 2B)

The same assumptions and definitions apply as in clause 4.5.2.

#### 4.5.2B.1	Initial conditions

System Simulator:

-	Cell 1

-	Cell 1 is transmitting SystemInformationBlockType8

User Equipment:

- 	The Test USIM shall be inserted.

#### 4.5.2B.2	Definition of system information messages

Table 4.5.2B.2-1: SystemInformationBlockType1 for cell 1 (Steps 1-41, Table 4.5.2B.3-2)

| Derivation Path: 36.508 Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| schedulingInformation ::= SEQUENCE (SIZE (1..maxSI-Message)) OF SEQUENCE {} | Combination 6 in TS 36.508 section 4.4.3.1 | SIB2, SIB3 and SIB 8 are transmitted |  |
| } |  |  |  |

Table 4.5.2B.2-2: SystemInformationBlockType8 for cell 1 (Steps 1-41, Table 4.5.2B.3-2)

| Derivation Path: 36.508 Table 4.4.3.3-7, condition HRPD |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType8 ::= SEQUENCE { |  |  |  |
| parametersHRPD SEQUENCE { |  |  |  |
| preRegistrationInfoHRPD SEQUENCE { |  |  |  |
| preRegistrationAllowed | TRUE |  |  |
| preRegistrationZoneId | ColorCode |  |  |
| secondaryPreRegistrationZoneIdList SEQUENCE (SIZE (1..2)) OF SEQUENCE { | Not present |  |  |
| PreRegistrationZoneIdHRPD | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| cellReselectionParametersHRPD SEQUENCE { |  |  |  |
| bandClassList SEQUENCE (SIZE (1..maxCDMA -BandClass)) OF SEQUENCE { | 1 entry |  |  |
| bandClass | Band Class of frequency under test | ENUMERATED {bc0, bc1, bc2, bc3, bc4, bc5, bc6, bc7, bc8, bc9, bc10, bc11, bc12, bc13, bc14, bc15, bc16, bc17, spare14, spare13, spare12, spare11, spare10, spare9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1, ...} |  |
| cellReselectionPriority | 3 |  |  |
| threshX-High | 30(-30) | INTEGER (0..63) |  |
| threshX-Low | 32(-32) | INTEGER (0..63) |  |
| } |  |  |  |
| neighCellList SEQUENCE (SIZE (1..16)) OF SEQUENCE { | 1 entry |  |  |
| bandClass | Band Class of frequency under test | ENUMERATED {bc0, bc1, bc2, bc3, bc4, bc5, bc6, bc7, bc8, bc9, bc10, bc11, bc12, bc13, bc14, bc15, bc16, bc17, spare14, spare13, spare12, spare11, spare10, spare9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1, ...} |  |
| } |  |  |  |
| } |  |  |  |
| t-ReselectionCDMA2000 | 3 | INTEGER (0..7) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 4.5.2B.3	Procedure

Table 4.5.2B.3-1 shows the cell power levels during the procedure.

Table 4.5.2B.3-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 |  | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -75 |  | - |

Table 4.5.2B.3-2: UE registration with HRPD pre-registration procedure (state 1 to state 2B)

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 to 16 | Same procedure for steps 1 to 16 as specified in the procedure in clause 4.5.2.3 | - | - |
| - | EXCEPTION: IF MULTI_PDN = TRUE THEN in parallel to the events described in steps 17 to 40 below, the generic procedure for UE establishing additional PDN connectivity as specified in TS 36.508 subclause 4.5A.16 takes place |  |  |
| 17 | The UE transmits an ULInformationTransfer containing a tunneled UATIRequest message Cell 1. | --> | ULInformationTransfer |
| 18 | The SS transmits a DLInformationTransfer containing a tunneled UATIAssignment message on Cell 1. | <-- | DLInformationTransfer |
| 19 | The UE transmits an ULInformationTransfer containing a tunneled UATIComplete message on Cell 1. | --> | ULInformationTransfer |
| 20 | The UE transmits an ULInformationTransfer containing a tunneled AlternateLinkOpenReqmessage on Cell 1. | --> | ULInformationTransfer |
| 21 | The SS transmits a DLInformationTransfer containing a tunneled AlternateLinkOpenConfmessage on Cell 1. | <-- | DLInformationTransfer |
| 22 | The UE transmits an ULInformationTransfer containing a tunneled AlternateLinkOpenCompleteon Cell 1. | --> | ULInformationTransfer |
| 23 | The UE transmits an ULInformationTransfer containing a tunneled ConfigurationRequest message for SCP configuration on Cell 1. | --> | ULInformationTransfer |
| 24 | The SS transmits a DLInformationTransfer containing a tunneled ConfigurationResponse message for SCP configuration on Cell 1. | <-- | DLInformationTransfer |
| 25 | The UE transmits an ULInformationTransfer containing a tunneled ConfigurationRequest message for Stream protocol on Cell 1. | --> | ULInformationTransfer |
| 26 | The SS transmits a DLInformationTransfer containing a tunneled ConfigurationResponse message for Stream protocol accepting EMPA bound to service network on Cell 1. | <-- | DLInformationTransfer |
| 27 | The UE transmits an ULInformationTransfer containing a tunneled EMPA ConfigurationRequest message or MMPA ConfigurationRequest message on Cell 1. | --> | ULInformationTransfer |
| 28 | The SS transmits a DLInformationTransfer containing a tunneled EMPA ConfigurationResponse message or MMPA ConfigurationResponse message on Cell 1. | <-- | DLInformationTransfer |
| 29 | The UE transmits an ULInformationTransfer containing a tunneled ConfigurationComplete message on Cell 1. | --> | ULInformationTransfer |
| 30 | The SS transmits a DLInformationTransfer containing a tunneled EMPA ConfigurationRequest message or MMPA ConfigurationRequest message on Cell 1. | <-- | DLInformationTransfer |
| 30A | The UE transmits an ULInformationTransfer containing a tunneled EMPA ConfigurationResponse message or MMPA ConfigurationResponse message on Cell 1. | --> | ULInformationTransfer |
| 30B | The SS transmits an DLInformationTransfer containing a tunneled SoftConfigurationComplete message on Cell 1. | <-- | DLInformationTransfer |
| 30C | The UE transmits an ULInformationTransfer containing a tunneled AlternateLinkCloseReq message on Cell 1. | --> | ULInformationTransfer |
| 30D | The SS transmits a DLInformationTransfer containing a tunneled AlternateLinkCloseConf message on Cell 1. | <-- | DLInformationTransfer |
| 31 | Optionally tunneled device level authentication may take place on Cell 1. | <--> | - |
| 32 | Optionally tunneled Location Update procedure may take place if the SS is configured to support it. | <--> | - |
| 32A | The UE transmits an ULInformationTransfer containing a tunneled AlternateLinkOpenReq message on Cell 1. | --> | ULInformationTransfer |
| 32B | The SS transmits a DLInformationTransfer containing a tunneled AlternateLinkOpenConf message on Cell 1. | <-- | DLInformationTransfer |
| 32C | The UE transmits an ULInformationTransfer containing a tunneled AlternateLinkOpenComplete message on Cell 1. | --> | ULInformationTransfer |
| 33 | Tunneled PPP LCP negotiation is performed between the UE and the SS. EAP-AKA’ is selected as the authentication protocol. | <--> | - |
| 33A | After entering PPP LCP Open State, optionally tunneled PPP Version Capability Indicaiton and/or Max PPP Inactivity Timer negotiation may take place. | <-> | - |
| 34 | Tunneled EAP-AKA’ is performed between the UE and the SS. | <--> | - |
| 35 | The UE transmits an ULInformationTransfer containing a tunneled VSNCP Configure-Request message, including a PDN-ID, PDN Type, APN, PDN Address, Protocol Configuration Options, and Attach Type = “handover”. The Address Allocation Preference option contained in the Protocol Configuration Options indicates whether the UE wants to perform the IP address allocation during the attach procedure or deferred IP address allocation. PDN Type indicates the UE’s IP capability (IPv4, IPv6 or IPv4/v6) | --> | ULInformationTransfer |
| 36 | The SS transmits a DLInformationTransfer containing a tunneled VSNCP Configure-Ack message. | <-- | DLInformationTransfer |
| 37 | The SS transmits a DLInformationTransfer containing a tunneled VSNCP Configure-Request message including the PDN-ID configuration option. | <-- | DLInformationTransfer |
| 38 | The UE transmits an ULInformationTransfer containing a tunneled VSNCP Configure-Ack message. | --> | ULInformationTransfer |
| 39 | Optionally deferred IP address allocation using tunnelled IETF messages can take place depending on the Address Allocation Preference indicated by the UE at Step 35. | <--> |  |
| 39A | The UE transmits an ULInformationTransfer containing a tunneled AlternateLinkCloseReq message on Cell 1. | --> | ULInformationTransfer |
| 39B | The SS transmits a DLInformationTransfer containing a tunneled AlternateLinkCloseConf message on Cell 1. | <-- | DLInformationTransfer |
| 40 | Void |  |  |
| 41 | The SS transmits an RRCConnectionRelease message on Cell 1. | <-- | RRCConnectionRelease |
| - | EXCEPTION: Steps 42a1 to 42b1 describe behaviour that depends on UE capabilities/configuration; the "lower case letter" identifies a step sequence that takes place if one of those is supported/configured. | - | - |
| 42a1 | IF MULTI_PDN AND pc_UE_supports_user_initiated_PDN_disconnect THEN the non-IMS PDN shall be released as specified in TS 36.508 subclause 4.5A.17 | - | - |
| 42b1 | IF MULTI_PDN AND NOT pc_UE_supports_user_initiated_PDN_disconnect THEN the non-IMS PDN shall be released as specified in TS 36.508 subclause 4.5A.18 | - | - |
| Note 1: The sequence in steps 17-40 lists important eHRPD procedures. More protocol negotiations can happen based on UE capability and SS emulation. Note 2: Message contents of tunnelled CDMA2000 messages (steps 17-40) are not checked in terms of conformance testing. |  |  |  |

NOTE: If step 31 is performed, an alternative link establishement procedure is also performed.

#### 4.5.2B.4	Specific message contents

All specific message contents shall be referred to clause 4.6, 4.7 and 4.7A with the exceptions below.

Table 4.5.2B.4-1: RRCConnectionRequest (Step 2, Table 4.5.2B.3-2)

| Derivation Path: Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionRequest ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcConnectionRequest-r8 SEQUENCE { |  |  |  |  |
| ue-Identity |  | Any allowed value |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 4.5.2B.4-2: UECapabilityInformation (Step13, Table 4.5.2B.3-2)

| Derivation Path: Table 4.6.1-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UECapabilityInformation ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| ueCapabilityInformation-r8 |  | Any allowed value |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 4.5.2B.4-3: UATI Request message (Step 17, Table 4.5.2B.3-2)

| Field | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| SAPState | ‘0’B | SAP Header |  |
| SessionConfigurationToken | 16 bits, Set by UE |  |  |
| ConnectionLayerFormat | 1 bit, Set by UE |  |  |
| ATI Record | 34 bits, RATI set by UE |  |  |
| Reserved | ‘0000’B |  |  |
| MessageID | ‘00000000’B | UATI Request |  |
| TransactionID | 8 bits, Set by UE |  |  |

Table 4.5.2B.4-4: UATI Assignment message (Step 18, Table 4.5.2B.3-2)

| Field | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| SAPState | ‘1’B | SAP Header |  |
| SessionConfigurationToken | ‘0’ |  |  |
| ConnectionLayerFormat | 1 bit, Set by SS |  |  |
| ATI Record | 34 bits, RATI set in UATI Request |  |  |
| Reserved | ‘0000’B |  |  |
| MessageID | ‘00000001’B | UATI Assignment |  |
| MessageSequence | 8 bits, Set by SS |  |  |
| Reserved1 | ‘0000000’B |  |  |
| SubnetIncluded | ‘1’B |  |  |
| UATISubnetMask | ‘1101000’B |  |  |
| UATI104 | 104 bits, Set by SS |  |  |
| UATIColorCode | 8 bits, Set to ColorCode |  |  |
| UATI024 | 24 bits, Set by SS |  |  |
| UpperOldUATILength | ‘0000’B |  |  |
| Reserved2 | ‘0000’B |  |  |

### 4.5.2C	UE Registration, pre-registration on 1xRTT (State 2C)

The same assumptions and definitions apply as in clause 4.5.2.

#### 4.5.2C.1	Initial conditions

System Simulator:

-	Cell 1

-	Cell 1 is transmitting SystemInformationBlockType8

User Equipment:

- 	The Test USIM shall be inserted.

-	The UE is switched OFF.

#### 4.5.2C.2	Definition of system information messages

Table 4.5.2C.2-1: SystemInformationBlockType1 for cell 1 (Steps 1-19, Table 4.5.2C.3-2)

| Derivation Path: 36.508 Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| schedulingInformation ::= SEQUENCE (SIZE (1..maxSI-Message)) OF SEQUENCE {} | Combination 6 in TS 36.508 section 4.4.3.1 | SIB2, SIB3 and SIB 8 are transmitted |  |
| } |  |  |  |

Table 4.5.2C.2-2: SystemInformationBlockType8 for cell 1 (Steps 1-19, Table 4.5.2C.3-2)

| Derivation Path: 36.508 Table 4.4.3.3-7, condition 1XRTT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType8 ::= SEQUENCE { |  |  |  |
| parameters1XRTT SEQUENCE { |  |  |  |
| csfb-RegistrationParam1XRTT SEQUENCE { |  |  |  |
| Sid | SID under test For Signalling test cases, see table 4.4.2-4 | BIT STRING (SIZE (15)) |  |
| Nid | NID under test For Signalling test cases, see table 4.4.2-4 | BIT STRING (SIZE (16)) |  |
| multipleSID | TRUE | BOOLEAN |  |
| multipleNID | TRUE | BOOLEAN |  |
| homeReg | TRUE | BOOLEAN |  |
| foreignSIDReg | TRUE | BOOLEAN |  |
| foreignNIDReg | TRUE | BOOLEAN |  |
| parameterReg | TRUE | BOOLEAN |  |
| powerUpReg | TRUE | BOOLEAN |  |
| registrationPeriod | ’0000000’ | BIT STRING (SIZE (7)) |  |
| registrationZone | Registration zone under test For Signalling test cases, see table 4.4.2-4 | BIT STRING (SIZE (12)) |  |
| totalZone | '001'B | BIT STRING (SIZE (3)) |  |
| zoneTimer | '000'B or '001'B | BIT STRING (SIZE (3)) |  |
| } |  |  |  |
| longCodeState1XRTT | A valid value, calculated and updated by the SS | BIT STRING (SIZE (42)) OPTIONAL |  |
| cellReselectionParameters1XRTT SEQUENCE { |  |  |  |
| bandClassList SEQUENCE (SIZE (1..maxCDMA -BandClass)) OF SEQUENCE { | 1 entry |  |  |
| bandClass | Band Class of frequency under test | ENUMERATED {bc0, bc1, bc2, bc3, bc4, bc5, bc6, bc7, bc8, bc9, bc10, bc11, bc12, bc13, bc14, bc15, bc16, bc17, spare14, spare13, spare12, spare11, spare10, spare9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1, ...} |  |
| cellReselectionPriority | 3 |  |  |
| threshX-High | 30(-30) | INTEGER (0..63) |  |
| threshX-Low | 32(-32) | INTEGER (0..63) |  |
| } |  |  |  |
| } |  |  |  |
| csfb-RegistrationParam1XRTT-v920{ |  |  |  |
| powerDownReg-r9 | TRUE |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 4.5.2C.3	Procedure

Table 4.5.2C.3-1 shows the cell power levels after the preamble.

Table 4.5.2C.3-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 |  | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -60 |  | - |

Table 4.5.2C.3-2: UE registration with 1xRTT pre-registration procedure (state 1 to state 2C)

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 to 16 | Same procedure for steps 1 to 16 as specified in the procedure in clause 4.5.2.3 | - | - |
| - | EXCEPTION: IF MULTI_PDN = TRUE THEN in parallel to the events described in steps 16A to 18 below, the generic procedure for UE establishing additional PDN connectivity as specified in TS 36.508 subclause 4.5A.16 takes place |  |  |
| 16A | The UE transmits a CSFBParametersRequestCDMA2000 on Cell 1? | --> | CSFBParametersRequestCDMA2000 |
| 16B | The SS transmits a CSFBParametersResponseCDMA2000 on Cell 1. | <-- | CSFBParametersResponseCDMA2000 |
| 17 | The UE transmits an ULInformationTransfer containing a 1xRTT GCSNA Encapsulated Registration message on Cell 1. | --> | ULInformationTransfer |
| 17A | The SS transmits a DLInformationTransfer containing a GCSNA Ack message on Cell 1. | <-- | DLInformationTransfer |
| 18 | The SS transmits a DLInformationTransfer containing a 1x RTT GCSNA Encapsulated Registration Accept Order on Cell 1. | <-- | DLInformationTransfer |
| 19 | The SS transmits an RRCConnectionRelease message on Cell 1. | <-- | RRCConnectionRelease |
| - | EXCEPTION: Steps 20a1 to 20b1 describe behaviour that depends on UE capabilities/configuration; the "lower case letter" identifies a step sequence that takes place if one of those is supported/configured. | - | - |
| 20a1 | IF MULTI_PDN AND pc_UE_supports_user_initiated_PDN_disconnect THEN the non-IMS PDN shall be released as specified in TS 36.508 subclause 4.5A.17 | - | - |
| 20b1 | IF MULTI_PDN AND NOT pc_UE_supports_user_initiated_PDN_disconnect THEN the non-IMS PDN shall be released as specified in TS 36.508 subclause 4.5A.18 | - | - |
| Note: Message contents of tunnelled CDMA2000 messages (steps 17-18) are not checked in terms of conformance testing. |  |  |  |

Table 4.5.2C.3-3: Void

#### 4.5.2C.4	Specific message contents

All specific message contents shall be referred to clause 4.6, 4.7 and 4.7A with the exceptions below.

Table 4.5.2C.4-1: RRCConnectionRequest (Step 2, Table 4.5.2C.3-2)

| Derivation Path: Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionRequest ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcConnectionRequest-r8 SEQUENCE { |  |  |  |  |
| ue-Identity |  | Any allowed value |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 4.5.2C.4-2: UECapabilityInformation (Step 13, Table 4.5.2C.3-2)

| Derivation Path: Table 4.6.1-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UECapabilityInformation ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| ueCapabilityInformation-r8 |  | Any allowed value |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 4.5.2C.4-3: Void

Table 4.5.2C.4-4: Void

Table 4.5.2C.4-5: CSFBParametersResponseCDMA2000 (Step 16B, Table 4.5.2C.3-2)

| Derivation Path: 36.508 Table 4.6.1-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSFBParametersResponseCDMA2000 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| csfbParametersResponseCDMA2000-r8 SEQUENCE { |  |  |  |
| Rand | Random Challenge Data set by SS |  |  |
| mobilityParameters | Set according to Table 4.5.2C.4-6 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 4.5.2C.4-6: mobilityParameters (CDMA2000Parameters) (Step 16B, Table 4.5.2C.3-2)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| RecordType | ‘00000000’B |  |  |
| SIDIncluded | ‘1’B |  |  |
| SID | The same as SIB8 |  |  |
| NIDIncluded | ‘1’B |  |  |
| NID | The same as SIB8 |  |  |
| REG_ZONEIncluded | ‘1’B |  |  |
| REG_ZONE | The same as SIB8 |  |  |
| TOTAL_ZONESIncluded | ‘1’B |  |  |
| TOTAL_ZONES | The same as SIB8 |  |  |
| ZONE_TIMERIncluded | ‘1’B |  |  |
| ZONE_TIMER | The same as SIB8 |  |  |
| PACKET_ZONE_IDIncluded | ‘0’B |  |  |
| PACKET_ZONE_ID | Not present |  |  |
| PZIDHystParametersIncluded | ‘0’B |  |  |
| PZ_HYST_ENABLED | Not present |  |  |
| PZ_HYST_INFO_INCL | Not present |  |  |
| PZ_HYST_LIST_LEN | Not present |  |  |
| PZ_HYST_ACT_TIMER | Not present |  |  |
| PZ_HYST_TIMER_MUL | Not present |  |  |
| PZ_HYST_TIMER_EXP | Not present |  |  |
| P_REVIncluded | ‘1’B |  |  |
| P_REV | ’00000110’B |  |  |
| MIN_P_REVIncluded | ‘1’B |  |  |
| MIN_P_REV | ’00000110’B |  |  |
| ‘0’B NEG_SLOT_CYCLE_INDEX_SUPIncluded |  |  |  |
| NEG_SLOT_CYCLE_INDEX_SUP | Not present |  |  |
| ENCRYPT_MODEIncluded | ‘0’B |  |  |
| ENCRYPT_MODE | Not present |  |  |
| ENC_SUPPORTEDIncluded | ‘0’B |  |  |
| ENC_SUPPORTED | Not present |  |  |
| SIG_ENCRYPT_SUPIncluded | ‘0’B |  |  |
| SIG_ENCRYPT_SUP | Not present |  |  |
| MSG_INTEGRITY_SUPIncluded | ‘0’B |  |  |
| MSG_INTEGRITY_SUP | Not present |  |  |
| SIG_INTEGRITY_SUP_INCLIncluded | ‘0’B |  |  |
| SIG_INTEGRITY_SUP_INCL | Not present |  |  |
| SIG_INTEGRITY_SUPIncluded | ‘0’B |  |  |
| SIG_INTEGRITY_SUP | Not present |  |  |
| AUTHIncluded | ‘1’B |  |  |
| AUTH | ‘00’B |  |  |
| MAX_NUM_ALT_SOIncluded | ‘1’B |  |  |
| MAX_NUM_ALT_SO | ‘000’B |  |  |
| USE_SYNC_IDIncluded | ‘0’B |  |  |
| USE_SYNC_ID | Not present |  |  |
| MS_INIT_POS_LOC_SUP_INDIncluded | ‘0’B |  |  |
| MS_INIT_POS_LOC_SUP_IND | Not present |  |  |
| MOB_QOSIncluded | ‘0’B |  |  |
| MOB_QOS | Not present |  |  |
| BAND_CLASS_INFO_REQIncluded | ‘0’B |  |  |
| BAND_CLASS_INFO_REQ | Not present |  |  |
| BAND_CLASSIncluded | ‘1’B |  |  |
| BAND_CLASS | ‘00000’B |  |  |
| BYPASS_REG_INDIncluded | ‘0’B |  |  |
| BYPASS_REG_IND | Not present |  |  |
| ‘0’B ALT_BAND_CLASSIncluded |  |  |  |
| ALT_BAND_CLASS | Not present |  |  |
| MAX_ADD_SERV_INSTANCEIncluded | ‘0’B |  |  |
| MAX_ADD_SERV_INSTANCE | Not present |  |  |
| HOME_REGIncluded | ‘1’B |  |  |
| HOME_REG | The same as SIB8 |  |  |
| FOR_SID_REGIncluded | ‘1’B |  |  |
| FOR_SID_REG | The same as SIB8 |  |  |
| FOR_NID_REGIncluded | ‘1’B |  |  |
| FOR_NID_REG | The same as SIB8 |  |  |
| POWER_UP_REGIncluded | ‘1’B |  |  |
| POWER_UP_REG | The same as SIB8 |  |  |
| POWER_DOWN_REGIncluded | ‘1’B |  |  |
| POWER_DOWN_REG | The same as SIB8 |  |  |
| PARAMETER_REGIncluded | ‘1’B |  |  |
| PARAMETER_REG | The same as SIB8 |  |  |
| REG_PRDIncluded | ‘1’B |  |  |
| REG_PRD | The same as SIB8 |  |  |
| REG_DISTIncluded | ‘0’B |  |  |
| REG_DIST | Not present |  |  |
| PREF_MSID_TYPEIncluded | ‘1’B |  |  |
| PREF_MSID_TYPE | ’11’B |  |  |
| EXT_PREF_MSID_TYPEIncluded | ‘0’B |  |  |
| EXT_PREF_MSID_TYPE | Not present |  |  |
| MEID_REQDIncluded | ‘0’B |  |  |
| MEID_REQD | Not present |  |  |
| MCCIncluded | ‘1’B |  |  |
| MCC | See the Table 4.4.2-4 Note 1, Note 2 |  |  |
| IMSI_11_12Included | ‘1’B |  |  |
| IMSI_11_12 | See the Table 4.4.2-4 Note 1, Note 2 |  |  |
| IMSI_T_SUPPORTEDIncluded | ‘1’B |  |  |
| IMSI_T_SUPPORTED | ’0’B |  |  |
| RECONNECT_MSG_INDIncluded | ‘0’B |  |  |
| RECONNECT_MSG_IND | Not present |  |  |
| RER_MODE_SUPPORTEDIncluded | ‘0’B |  |  |
| RER_MODE_SUPPORTED | Not present |  |  |
| TKZ_MODE_SUPPORTEDIncluded | ‘0’B |  |  |
| TKZ_MODE_SUPPORTED | Not present |  |  |
| TKZ_IDIncluded | ‘0’B |  |  |
| TKZ_ID | Not present |  |  |
| PILOT_REPORTIncluded PILOT_REPORT | ‘0’B |  |  |
| PILOT_REPORT | Not present |  |  |
| SDB_SUPPORTEDIncluded | ‘0’B |  |  |
| SDB_SUPPORTED | Not present |  |  |
| AUTO_FCSO_ALLOWEDIncluded | ‘0’B |  |  |
| AUTO_FCSO_ALLOWED | Not present |  |  |
| SDB_IN_RCNM_INDIncluded | ‘0’B |  |  |
| SDB_IN_RCNM_IND | Not present |  |  |
| FPC_FCH_Included | ‘1’B |  |  |
| FPC_FCH_INIT_SETPT_RC3 | ’00011000’B |  |  |
| FPC_FCH_INIT_SETPT_RC4 | ’00011000’B |  |  |
| FPC_FCH_INIT_SETPT_RC5 | ’00011000’B |  |  |
| FPC_FCH_INIT_SETPT_RC11 | ’00000000’B |  |  |
| FPC_FCH_INIT_SETPT_RC12 | ’00000000’B |  |  |
| T_ADD_Included | ‘0’B |  |  |
| T_ADD | Not present |  |  |
| PILOT_INC_Included | ‘1’B |  |  |
| PILOT_INC | ’0010’B |  |  |
| RAND_Included | ‘0’B |  |  |
| RAND | Not present |  |  |
| LP_SEC_Included | ‘1’B |  |  |
| LP_SEC | ‘00000000’B |  |  |
| LTM_OFF_Included | ‘1’B |  |  |
| LTM_OFF | ‘000000’B |  |  |
| DAYLT_Included | ‘1’B |  |  |
| DAYLT | ‘0’B |  |  |
| GCSNAL2AckTimer_Included | ‘1’B |  |  |
| GCSNAL2AckTimer | ‘00000001’B |  |  |
| GCSNASequenceContextTimer_Incl uded | ‘1’B |  |  |
| GCSNASequenceContextTimer | ‘00000101’B |  |  |

Table 4.5.2C.4-7: ULInformationTransfer (Step 17, Table 4.5.2C.3-2)

| Derivation Path: 36.508 Table 4.6.1-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| ULInformationTransfer ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| ulInformationTransfer-r8 SEQUENCE { |  |  |  |  |
| dedicatedInformationType CHOICE { |  |  |  |  |
| dedicatedInfoCDMA2000-1XRTT |  | Set according to Table 4.5.2C.4-8 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 4.5.2C.4-8: 1xRTT GCSNA Encapsulated Registration message (Step 17, Table 4.5.2C.3-2)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| MessageID | ‘00000001’B | GCSNA1xCircuitService message |  |
| GCSNAOption | ‘00000001’B |  |  |
| AlternativeGCSNAOption_INCL | ‘0’B |  |  |
| IWSIDIncl | ‘0’B |  |  |
| AckRequired | ‘1’B |  |  |
| StopDupDetect | ‘0’B |  |  |
| MessageSequence | Set by UE |  |  |
| NumTLACEncapsulated1xL3PDU | ‘00’B |  |  |
| Reserved | ‘0000’B |  |  |
| 1xLogicalChannel | ‘0’B |  |  |
| 1xProtocolRevision | ’00000110’B |  |  |
| MsgType | ‘00000001’B | Registration message |  |
| NumTLACHeaderRecords | ‘0001’B |  |  |
| TLACHeaderRecordType | ‘0000’B |  |  |
| TLACHeaderRecordLength | 8 bits, Set by UE |  |  |
| MSID_TYPE | ‘011’B |  |  |
| MSID_LEN | 4 bits, Set by UE |  |  |
| MSID | Variable, Set by UE |  |  |
| Reserved | ‘0000000’B |  |  |
| 1xL3PDULength | 16 bits, Set by UE |  |  |
| REG_TYPE | ‘0001’B | Power up registration |  |
| SLOT_CYCLE_INDEX | ‘010’B |  |  |
| MOB_P_REV | 8 bits, Set by UE |  |  |
| SCM | 8 bits, Set by UE |  |  |
| MOB_TERM | ‘1’B |  |  |
| RETURN_CAUSE | ‘1’B |  |  |
| QPCH_SUPPORTED | ‘1’B |  |  |
| ENHANCED_RC | ‘1’B |  |  |
| UZID_INCL | ‘0’B |  |  |

Table 4.5.2C.4-9: 1xRTT CS Registration Accept Order (Step 18, Table 4.5.2C.3-2)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| MessageID | ‘00000001’B |  |  |
| GCSNAOption | ‘00000001’B |  |  |
| AlternativeGCSNAOption_INCL | ‘0’B |  |  |
| IWSIDIncl | ‘0’B |  |  |
| AckRequired | ‘0’B |  |  |
| StopDupDetect | ‘0’B |  |  |
| MessageSequence | 6 bits, Set by SS |  |  |
| NumTLACEncapsulated1xL3PDU | ‘00’B |  |  |
| Reserved | ‘0000’B |  |  |
| 1xLogicalChannel | ‘0’B |  |  |
| 1xProtocolRevision | ’00000110’B |  |  |
| MsgType | ‘00000111’B | Order message |  |
| NumTLACHeaderRecords | ‘0000’B |  |  |
| Reserved | ‘000’B |  |  |
| 1xL3PDULength | 16 bits, Set by SS |  |  |
| ORDER | ‘011011’B | Registration Accept Order |  |
| ADD_RECORD_LEN | ‘001’B |  |  |
| ORDQ | ‘00000000’B |  |  |

Table 4.5.2C.4-10: ATTACH ACCEPT (Step 14, Table 4.5.2C.3-2)

| Derivation Path: 36.508 clause 4.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| EPS attach result |  | '0001'B | EPS only |  |
| EMM cause |  | '0001 0010'B | #18 “CS domain not available” | combined_EPS_IMSI |

### 4.5.2D	UE Registration, 2 PDN for RAN Assisted WLAN Interworking (State 2)

The same assumptions and definitions apply as in clause 4.5.2.

#### 4.5.2D.1	Initial conditions

System Simulator:

-	1 cell, default parameters.

-	The procedure shall be performed under ideal radio conditions as defined in clause 5

User Equipment:

-	The Test UICC shall be inserted. This shall contain either ISIM and USIM applications or only a USIM application on UICC.

#### 4.5.2D.2	Definition of system information messages

The default system information messages are used.

#### 4.5.2D.3	Procedure

Table 4.5.2D.3-1: UE registration procedure (state 1 to state 2)

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1-16 | Steps 1 to 16 of the generic test procedure for UE registration sub clause 4.5.2.3. | - | - |
|  | EXCEPTION: Step17 depends on UE configuration, executed if UE does not automatically trigger additional PDN Connection. |  |  |
| 17 | Cause the UE to request connectivity to an additional PDN (see Note 1) |  |  |
| 18-21 | The generic procedure for UE establishing additional PDN connectivity as specified in TS 36.508 subclause 4.5A.16 takes place |  |  |
| 22 | The SS transmits an RRCConnectionRelease message to release RRC connection and move to RRC_IDLE (State 2). | <-- | RRC: RRCConnectionRelease |
| NOTE 1: The request of connectivity to an additional PDN may be performed by MMI or AT command. |  |  |  |

#### 4.5.2D.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7 with the exceptions below.

Table 4.5.2D.4-1: RRCConnectionRequest (Step 2)

| Derivation Path: Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionRequest ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcConnectionRequest-r8 SEQUENCE { |  |  |  |  |
| ue-Identity |  | Any allowed value |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 4.5.2D.4-2: UECapabilityInformation (Step 13)

| Derivation Path: Table 4.6.1-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UECapabilityInformation ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| ueCapabilityInformation-r8 |  | Any allowed value |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 4.5.2D.4-3: ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST (Step 14 a)

| Derivation Path: Table 4.7.3-6 with condition IMS_PDN_ConnEstab for PDN1_IMS and NOT IMS_PDN_ConnEstab else |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| EPS bearer identity |  | '0101'B | arbitrary value used for PDN connectivity being maintained during the test case | PDN1_IMS OR NOT MULTI_PDN |
| EPS bearer identity |  | '1100'B | arbitrary value used for additional PDN connectivity which may be released before entering the test procedure of the test case | MULTI_PDN AND NOT PDN1_IMS |
| PDN address |  |  |  | IPv4_address_only |
| Length of PDN address contents |  | 5 octets |  |  |
| PDN type value |  | ‘001’B | IPv4 |  |
| PDN address information |  | IPv4 address | The SS provides a valid IPv4 address | NOT IPv4-DHCP |
|  |  | 0.0.0.0 | DHCPv4 is to be used to allocate the IPv4 address | IPv4-DHCP |
| ESM cause |  | IF "PDN type" IE in step 4 is 'IPv4v6' THEN '00110010'B ELSE Not present | "PDN type IPv4 only allowed" |  |
| WLAN offload indication |  | Not present |  | NOT PDN1_Offload |
| WLAN offload indication |  |  |  | PDN1_Offload |
| WLAN offload acceptability |  | ‘0001’B | Offloading the traffic of the PDN connection via a WLAN when in S1 mode is acceptable |  |

NOTE:	The default message contents specified in table 4.7.3-6 apply unless the condition IPv4_address_only in table 4.5.2D.4-3 is true.

| Condition | Explanation |
| --- | --- |
| IPv4_address_only | This condition applies if the test case preamble description indicates that the UE is allocated an IPv4 address. |
| IPv4-DHCP | If in the last PDN CONNECTIVITY REQUEST or ESM INFORMATION RESPONSE sent prior to this message, the IE Protocol configuration options contains a configuration protocol option = '000B00H' ("IPv4 address allocation via DHCPv4", length of contents = 0). Note 1: This condition is used in conjunction with IPv4 or IPv4v6 as indicated in the "PDN address row" just above. Note 2: If both messages, PDN CONNECTIVITY REQUEST and ESM INFORMATION RESPONSE, are received and contain a Protocol configuration options IE then the IE from the message which is received later shall be used. |
| PDN1_Offload | This condition applies if the test execution requests the first PDN connection PDN1 to be offloaded to WLAN |

Table 4.5.2D.4-4: Message ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST (step 19)

| Derivation path: Table 4.7.3-6 with condition IMS_PDN_ConnEstab for ADD_IMS and NOT IMS_PDN_ConnEstab else and table 4.6.1-8 with condition AM-DRB-ADD(1) for ADD_IMS or with condition AM-DRB-ADD(8) else |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS bearer identity | '0101'B | arbitrary value used for PDN connectivity being maintained during the test case | PDN2_IMS |
| EPS bearer identity | '1100'B | arbitrary value used for additional non-IMS PDN connectivity | NOT PDN2_IMS |
| Procedure transaction identity | PTI-1 | SS re-uses the particular PTI defined by UE for this present additional PDN connectivity request procedure |  |
| WLAN offload indication | Not present |  | PDN1_Offload |
| WLAN offload indication |  |  | NOT PDN1_Offload |
| WLAN offload acceptability | ‘0001’B | Offloading the traffic of the PDN connection via a WLAN when in S1 mode is acceptable |  |

| Condition | Explanation |
| --- | --- |
| PDN1_Offload | This condition applies if the test execution requests the first PDN connection PDN1 to be offloaded to WLAN |

### 4.5.3	Generic Radio Bearer Establishment (State 3)

IF pc_ue_CategoryDL_M1 THEN use the procedure for State 3-CE according to clause 4.5.3AA ELSE use the procedure defined in the present clause.

NOTE:	The statement to use alternate procedure for CAT M1 UE (pc_ue_CategoryDL_M1 is set) is to enable LTE legacy protocol test cases to be used for CAT M1 UE testing. Any CAT M1 or CE specific test cases should not rely on this statement, but refer directly to the relevant CE specific state.

#### 4.5.3.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall be in Registered, Idle Mode state (State 2).

#### 4.5.3.2	Definition of system information messages

The default system information messages are used.

#### 4.5.3.3	Procedure

The establishment of generic radio bearer connection is assumed to always be mobile terminated.

Table 4.5.3.3-1: Generic Radio Bearer establishment procedure (state 2 to state 3)

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Void |  |  |
| 2 | SS sends a Paging message to the UE on the appropriate paging block, and including the UE identity in one entry of the IE pagingRecordLists. | <-- | RRC: Paging (PCCH) |
| 3 | UE transmits an RRCConnectionRequest message. | --> | RRC: RRCConnectionRequest |
| 4 | SS transmit an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| 5 | The UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the session management procedure by including the SERVICE REQUEST message. (State3) | --> | RRC: RRCConnectionSetupComplete NAS: SERVICE REQUEST |
| 6 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |
| 7 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 8 | The SS configures a new data radio bearer, associated with the default EPS bearer context. IF the test case using state 3 as an initial state is using N additional data radio bearers (0 ≤ N ≤ 7), the RRCConnectionReconfiguration message contains the configuration of these additional data radio bearers in the drb-ToAddModify IE and one ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message per additional data radio bearer. The RRCConnectionReconfiguration message is using condition SRB2-DRB(1+n, m) where n and m are the number of DRBs associated with dedicated EPS bearer contexts and configured respectively with RLC-AM and RLC-UM (n+m = N). | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST ... ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST |
| - | EXCEPTION: the steps 9 and 10a1 to 10a3 (if present) happen in any order. | - | - |
| 9 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer context. | --> | RRC: RRCConnectionReconfigurationComplete |
| 10a1 | IF N 1 (see step 8), the UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message matching with the 1st ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message included in step 8. | --> | RRC: ULInformationTransfer NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |
| 10a2 | IF N 2 (see step 8), the UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message matching with the 2nd ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message included in step 8. | --> | RRC: ULInformationTransfer NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |
| 10a3 | IF N 3 (see step 8), the UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message matching with the 3rd ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message included in step 8. | --> | RRC: ULInformationTransfer NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |
| 10a4 | IF N 4 (see step 8), the UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message matching with the 4th ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message included in step 8. | --> | RRC: ULInformationTransfer NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |
| 10a5 | IF N 5 (see step 8), the UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message matching with the 5th ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message included in step 8. | --> | RRC: ULInformationTransfer NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |
| 10a6 | IF N 6 (see step 8), the UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message matching with the 6th ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message included in step 8. | --> | RRC: ULInformationTransfer NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |
| 10a7 | IF N = 7 (see step 8), the UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message matching with the 7th ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message included in step 8. | --> | RRC: ULInformationTransfer NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |

#### 4.5.3.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7.

### 4.5.3A	Generic Radio Bearer Establishment, UE Test Mode Activated (State 3A)

IF pc_ue_CategoryDL_M1 THEN use the procedure for State 3A-CE according to clause 4.5.3AB ELSE use the procedure defined in the present clause.

NOTE:	The statement to use alternate procedure for CAT M1 UE (pc_ue_CategoryDL_M1 is set) is to enable LTE legacy protocol test cases to be used for CAT M1 UE testing. Any CAT M1 or CE specific test cases should not rely on this statement, but refer directly to the relevant CE specific state.

#### 4.5.3A.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall be in Registered, Idle Mode state, UE Test Mode Activated (State 2A for Signalling testing).

-	The UE shall be in Registered, Idle Mode state, UE Test Mode Activated (State 2A-RF for RF and RRM testing).

-	The UE shall be in Registered, Idle Mode state, UE Test Mode Activated (State 2A or State 2A-RF for UE positioning testing).

#### 4.5.3A.2	Definition of system information messages

The default system information messages are used.

#### 4.5.3A.3	Procedure

Same procedure as specified in the procedure in clause 4.5.3.3.

#### 4.5.3A.4	Specific message contents

All specific message contents shall be referred to clause 4.6, 4.7 and 4.7A.

### 4.5.3AA	Generic Radio Bearer Establishment (State 3-CE)

#### 4.5.3AA.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall be in Registered, Idle Mode state (State 2-CE).

#### 4.5.3AA.2	Definition of system information messages

The default system information messages as specified in clause 4.5.2AA.2.

#### 4.5.3AA.3	Procedure

Table 4.5.3AA.3-1: UE registration procedure (state 2-CE to state 3-CE)

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Void |  |  |
| 2 to 10a7 | Same procedure for steps 2 to 10a7 as specified in the procedure in clause 4.5.3.3. | - | - |

#### 4.5.3AA.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7 with the exception below:

-	In addition to any other conditions use the condition CEmodeB if the test case specifically tests CE mode B else use condition CEmodeA.

### 4.5.3AB	Generic Radio Bearer Establishment, UE Test Mode Activated (State 3A-CE)

#### 4.5.3AB.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall be in Registered, Idle Mode state, UE Test Mode Activated (State 2A-CE for Signalling testing).

-	The UE shall be in Registered, Idle Mode state, UE Test Mode Activated (State 2A-RF-CE for RF and RRM testing).

#### 4.5.3AB.2	Definition of system information messages

The default system information messages are used as specified in clause 4.5.2AA.2.

#### 4.5.3AB.3	Procedure

Table 4.5.3AB.3-1: UE registration procedure (State 2A-CE/2A-RF-CE to state 3A-CE)

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Void |  |  |
| 2 to 10a7 | Same procedure for steps 2 to 10a7 as specified in the procedure in clause 4.5.3.3 | - | - |

#### 4.5.3AB.4	Specific message contents

All specific message contents shall be referred to clause 4.6, 4.7 and 4.7A with the exception below:

-	In addition to any other conditions use the condition CEmodeB if the test case specifically tests CE mode B else use condition CEmodeA.

### 4.5.3B	Generic Radio Bearer Establishment, pre-registered on HRPD (State 3B)

#### 4.5.3B.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall be in Registered, Idle Mode state, pre-registered on HRPD (State 2B).

#### 4.5.3B.2	Definition of system information messages

The default system information messages are used.

#### 4.5.3B.3	Procedure

Same procedure as specified in the procedure in clause 4.5.3.3.

#### 4.5.3B.4	Specific message contents

All specific message contents shall be referred to clause 4.6, 4.7 and 4.7A.

### 4.5.3C	Generic Radio Bearer Establishment, pre-registered on 1xRTT (State 3C)

#### 4.5.3C.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall be in Registered, Idle Mode state, pre-registered on 1xRTT (State 2C).

#### 4.5.3C.2	Definition of system information messages

The default system information messages are used.

#### 4.5.3C.3	Procedure

Same procedure as specified in the procedure in clause 4.5.3.3.

#### 4.5.3C.4	Specific message contents

All specific message contents shall be referred to clause 4.6, 4.7 and 4.7A.

### 4.5.3D	Generic Radio Bearer Establishment for RAN Assisted WLAN Interworking (State 3)

#### 4.5.3D.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall be in Registered, Idle Mode state (State 2) as per 4.5.2D establishing 2 PDN connections.

#### 4.5.3D.2	Definition of system information messages

The default system information messages are used.

#### 4.5.3D.3	Procedure

The establishment of generic radio bearer connection is assumed to always be mobile terminated.

Table 4.5.3D.3-1: Generic Radio Bearer establishment procedure (state 2 to state 3)

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1-7 | Steps 1 to 16 of the generic test procedure for Radio Bearer Establishment sub clause 4.5.3.3 | - | - |
| 8 | The SS configures a 2 new data radio bearer, associated with the 2 default EPS bearer contexts. IF the test case using state 3 as an initial state is using N additional data radio bearers (0 ≤ N ≤ 7), the RRCConnectionReconfiguration message contains the configuration of these additional data radio bearers in the drb-ToAddModify IE and one ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message per additional data radio bearer. The RRCConnectionReconfiguration message is using condition SRB2-DRB(1+n, m) where n and m are the number of DRBs associated with dedicated EPS bearer contexts and configured respectively with RLC-AM and RLC-UM (n+m = N). | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST ... ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST |
| 9-10a7 | Steps 9 to 10a7 of the generic test procedure for Radio Bearer Establishment sub clause 4.5.3.3 | - | - |

#### 4.5.3D.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7.

### 4.5.3E	Control plane CIoT connection request (State 3-CP)

#### 4.5.3E.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

-	TS 36.508 condition CIoT-test apply.

User Equipment:

-	The UE shall be in Registered, Idle Mode state (State 2 or 2-CE).

#### 4.5.3E.2	Definition of system information messages

The default system information messages are used.

#### 4.5.3E.3	Procedure

The establishment of generic radio bearer connection is assumed to always be mobile terminated.

Table 4.5.3E.3-1: Generic control plane CIoT connection request procedure (state 3-CP)

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Void |  |  |
| 2 | SS sends a Paging message to the UE on the appropriate paging block, and including the UE identity in one entry of the IE pagingRecordLists. | <-- | RRC: Paging (PCCH) |
| 3 | UE transmits an RRCConnectionRequest message. | --> | RRC: RRCConnectionRequest |
| 4 | SS transmit an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| 5 | The UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the session management procedure by including the CONTROL PLANE SERVICE REQUEST message. | --> | RRC: RRCConnectionSetupComplete NAS: CONTROL PLANE SERVICE REQUEST |
| 6 | The SS transmits a SERVICE ACCEPT message. | <-- | RRC: DLInformationTransfer NAS: SERVICE ACCEPT |

#### 4.5.3E.4	Specific message contents

All specific message contents shall be referred to clauses 4.6 and 4.7.

### 4.5.3EA	Control plane CIoT connection request, UE Test Mode Activated (State 3A-CP)

#### 4.5.3EA.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

-	TS 36.508 condition CIoT-test apply.

User Equipment:

-	The UE shall be in Registered, Idle Mode state (State 2A or 2A-CE).

#### 4.5.3EA.2	Definition of system information messages

The default system information messages are used.

#### 4.5.3EA.3	Procedure

Same procedure as specified in the procedure in clause 4.5.3E.3.

#### 4.5.3EA.4	Specific message contents

All specific message contents shall be referred to clauses 4.6 and 4.7.

### 4.5.3F	User plane CIoT connection request (State 3-UP)

#### 4.5.3F.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

-	TS 36.508 conditions CIoT-test and UP-CIoT applies.

User Equipment:

-	The UE shall be in Registered, Idle Mode state (State 2 or 2-CE).

#### 4.5.3F.2	Definition of system information messages

The default system information messages are used.

#### 4.5.3F.3	Procedure

The establishment of generic radio bearer connection is assumed to always be mobile terminated.

Table 4.5.3F.3-1: Generic user plane CIoT connection request procedure (state 3-UP)

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Void |  |  |
| 2 | SS sends a Paging message to the UE on the appropriate paging block, and including the UE identity in one entry of the IE pagingRecordLists. | <-- | RRC: Paging (PCCH) |
| 3 | UE transmits an RRCConnectionResumeRequest message. | --> | RRC: RRCConnectionResumeRequest |
| 4 | SS transmit an RRCConnectionResume message. | <-- | RRC: RRCConnectionResume |
| 5 | The UE transmits an RRCConnectionResumeComplete message. | --> | RRC: RRCConnectionResumeComplete |

#### 4.5.3F.4	Specific message contents

All specific message contents shall be referred to clauses 4.6 and 4.7.

### 4.5.3FA	User plane CIoT connection request, UE Test Mode Activated (State 3A-UP)

#### 4.5.3FA.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

-	TS 36.508 conditions CIoT-test and UP-CIoT applies.

User Equipment:

-	The UE shall be in Registered, Idle Mode state (State 2 or 2-CE).

#### 4.5.3FA.2	Definition of system information messages

The default system information messages are used.

#### 4.5.3FA.3	Procedure

Same procedure as specified in the procedure in clause 4.5.3F.3.

#### 4.5.3FA.4	Specific message contents

All specific message contents shall be referred to clauses 4.6 and 4.7.

### 4.5.4	Loopback Activation (State 4)

IF pc_ue_CategoryDL_M1 THEN use the procedure for State 4-CE according to clause 4.5.4A ELSE use the procedure defined in the present clause.

NOTE:	The statement to use alternate procedure for CAT M1 UE (pc_ue_CategoryDL_M1 is set) is to enable LTE legacy protocol test cases to be used for CAT M1 UE testing. Any CAT M1 or CE specific test cases should not rely on this statement, but refer directly to the relevant CE specific state.

#### 4.5.4.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall be in Generic RB Establishment state, UE Test Mode Activated (State 3A).

#### 4.5.4.2	Definition of system information messages

The default system information messages are used.

#### 4.5.4.3	Procedure

Table 4.5: 4.3-1: Loopback activation procedure (state 3A to state 4)

| Step | Procedure | Direction | Message Sequence |
| --- | --- | --- | --- |
|  |  | UE - SS | Message |
| 1 | The SS transmits a CLOSE UE TEST LOOP message to enter the UE test loop mode. | <-- | RRC: DLInformationTransfer TC: CLOSE UE TEST LOOP |
| 2 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message to confirm that loopback entities for the radio bearer(s) have been created and loop back is activated (State 4). | --> | RRC: ULInformationTransfer TC: CLOSE UE TEST LOOP COMPLETE |

#### 4.5.4.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7A.

### 4.5.4A	Loopback Activation in cell supporting BL/CE UE (State 4-CE)

#### 4.5.4A.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall be in Generic RB Established, UE Test Mode Activated, Cell supporting BL/CE UE, UE Test Mode Activated (State 3A-CE).

#### 4.5.4A.2	Definition of system information messages

Same system information message content as in clause 4.5.2AA.2.

#### 4.5.4A.3	Procedure

Table 4.5.4A.3-1: Loopback activation procedure (state 2A-CE to state 4-CE)

| Step | Procedure | Direction | Message Sequence |
| --- | --- | --- | --- |
|  |  | UE - SS | Message |
| 1 | The SS transmits a CLOSE UE TEST LOOP message to enter the UE test loop mode. | <-- | RRC: DLInformationTransfer TC: CLOSE UE TEST LOOP |
| 2 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message to confirm that loopback entities for the radio bearer(s) have been created and loop back is activated (State 4). | --> | RRC: ULInformationTransfer TC: CLOSE UE TEST LOOP COMPLETE |

#### 4.5.4A.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7A.

### 4.5.4B	Loopback Activation user plane (State 4A-UP)

#### 4.5.4B.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

-	TS 36.508 conditions CIoT-test and UP-CIoT applies.

User Equipment:

-	User plane CIoT connection request, UE Test Mode Activated (State 3A-UP).

#### 4.5.4B.2	Definition of system information messages

The default system information messages are used.

#### 4.5.4B.3	Procedure

Same procedure as specified in the procedure in clause 4.5.4A.3.

#### 4.5.4B.4	Specific message contents

All specific message contents shall be referred to clauses 4.6 and 4.7.

### 4.5.5	HRPD registration (State H2)

Editor's note: The default parameter and system information will depend on progress in Clause 4.4, once it is finalised RAN5 can refer to them accordingly.

#### 4.5.5.1	Initial conditions

System Simulator:

-	Cell 1 and Cell 15.

-	Cell 1 has a lower reselection priority than Cell 15.

-	Cell 15 is transmitting default overhead messages.

User Equipment:

- 	The Test USIM shall be inserted (State 1).

#### 4.5.5.2	Definition of system information messages

The default overhead messages as specified in clause 4.4 are used.

#### 4.5.5.3	Procedure

Editor note: procedure needs to be specified.

#### 4.5.5.4	Specific message contents

Editor's note: message contents need to be specified where necessary.

### 4.5.5A	HRPD registration, pre-registration on E-UTRAN (State H2A)

Editor's note: The default parameter and system information will depend on progress in Clause 4.4, once it is finalised RAN5 can refer to them accordingly.

#### 4.5.5A.1	Initial conditions

System Simulator:

-	Cell 1 and Cell 15.

-	Cell 1 has a lower reselection priority than Cell 15.

-	Cell 15 is transmitting default overhead messages.

User Equipment:

- 	The Test USIM shall be inserted (State 1).

#### 4.5.5A.2	Definition of system information messages

The default overhead messages as specified in clause 4.4 are used.

#### 4.5.5A.3	Procedure

Editor's note: procedure needs to be specified.

#### 4.5.5A.4	Specific message contents

Editor's note: message contents need to be specified where necessary.

### 4.5.6	HRPD session establishment (State H3)

Editor's note: The default parameter and system information will depend on progress in Clause 4.4, once it is finalised RAN5 can refer to them accordingly.

#### 4.5.6.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall be in HRPD Registered, Dormant Mode state (State H2).

#### 4.5.6.2	Definition of system information messages

The default overhead messages as specified in clause 4.4 are used.

#### 4.5.6.3	Procedure

Editor's note: procedure needs to be specified.

#### 4.5.6.4	Specific message contents

Editor's note: message contents need to be specified where necessary.

### 4.5.6A	HRPD session establishment, pre-registered on E-UTRAN (State H3A)

Editor's note: The default parameter and system information will depend on progress in Clause 4.4, once it is finalised RAN5 can refer to them accordingly.

#### 4.5.6A.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall be in HRPD Registered, Dormant Mode, pre-registered on E-UTRAN state (State H2A).

#### 4.5.6A.2	Definition of system information messages

The default overhead messages as specified in clause 4.4 are used.

#### 4.5.6A.3	Procedure

Same procedure as specified in the procedure in clause 4.5.6.3

#### 4.5.6A.4	Specific message contents

Editor's note: message contents need to be specified where necessary.

### 4.5.7	Out of Coverage (State 5)

Out of coverage UE operation is applicable only for sidelink direct communication. In this case UE uses preconfigured radio parameters for transmission and reception of side link direct communication. UE remains in any cell selection state.

#### 4.5.7.1	Initial conditions

System Simulator:

-	1 cell, default parameters.

-	The procedure shall be performed under the condition that there is no reference signal and SIB transmission by the cell.

User Equipment:

-	The Test UICC shall be inserted. This shall contain either ISIM and USIM applications or only a USIM application on UICC.

#### 4.5.7.2	Definition of system information messages

MIB and SIBs are not transmitted.

#### 4.5.7.3	Procedure

UE remains in any cell selection state and search for any available cell.

### 4.5.8	Out of Coverage, V2X setup (State 5-V2X)

#### 4.5.8.1	Initial conditions

System Simulator:

-	No E-UTRA cell configured

-	1 GNSS simulator

-	1 simulated V2V UE (transmitter or receiver according to specific test case configuration)

User Equipment:

-	If supported by the UE, the Test UICC shall be inserted. This shall contain either ISIM and USIM applications or only a USIM application on UICC.

#### 4.5.8.2	Definition of system information messages

None.

#### 4.5.8.3	Procedure

UE remains in any cell selection state and search for any available cell.

### 4.5.9	Out of Coverage, Test Loopback Activation, V2X setup (State 5A-V2X)

The same assumptions and definitions apply as in clause 4.5.8. State 5A-V2X shall be invoked either in Receive Mode or in Transmit Mode, depending on the test purpose.

#### 4.5.9.1	Initial conditions

System Simulator:

-	No E-UTRA cell configured

-	1 GNSS simulator

-	1 simulated V2X UE (transmitter or receiver according to specific test case configuration)

User Equipment:

-	The UE shall be in Out of Coverage, V2X setup state (State 5-V2X)

#### 4.5.9.2	Definition of system information messages

None.

#### 4.5.9.3	Procedure

Table 4.5.9.3-1: Out of Coverage, Test Loopback Activation, V2X setup procedure (state 5-V2X to state 5A-V2X)

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Trigger UE to activate UE test loop mode. NOTE: The activation of UE test loop mode may be performed by MMI or AT command (+CATM). | - | - |
| 2A | Trigger UE to close UE test loop back mode E (Receive Mode). NOTE: Closing of UE test loop mode E may be performed by MMI or AT command (+CCUTLE) | - | - |
| 2B | Trigger UE to close UE test loop back mode E (Transmit Mode). NOTE: Closing of UE test loop mode E may be performed by MMI or AT command (+CCUTLE) | - | - |
| 3 | Trigger UE to reset or clear the current UTC time that has been calculated from GNSS. NOTE: The UTC time can be reset or clear on the UE using AT command (+CUTCR). | - | - |
| Note 1: If State 5A-V2X was invoked in Receive Mode use Step 2A; if State 5A-V2X was invoked in Transmit Mode use Step 2B. |  |  |  |

#### 4.5.9.4	Specific message contents

All specific message contents shall be referred to clause 4.7I with the exceptions below.

Table 4.5.9.4-1: AT COMMAND: +CATM (Step 1)

Derivation Path: Table 4.7I-1: +CATM with condition Activation

Table 4.5.9.4-2A: AT COMMAND: +CCUTLE (Step 2A)

Derivation Path: Table 4.7I-2: +CCUTLE with condition Close and Receive.

Table 4.5.9.4-2B: AT COMMAND: +CCUTLE (Step 2B)

Derivation Path: Table 4.7I-2: +CCUTLE with condition Close and Transmit.

Table 4.5.9.4-3: AT COMMAND: +CUTCR (Step 3)

Derivation Path: Table 4.7I-3: +CUTCR

## 4.5A	Other generic procedures

### 4.5A.1	Procedure for IP address allocation in the U-plane

The purpose of this procedure is to allow the successful completion of IP address allocation if it is initiated by the UE therefore the result from the execution of the Procedure for IP address allocation in the U-plane shall not lead to assignment of a verdict.

Depending on the UE configuration there may be unpredictable delay in the start of the procedure. A guarding time of 1.2 sec is suggested within which the procedure is expected to start. If the timer expires then the test procedure, from which the Procedure for IP address allocation in the U-plane is called, shall advance to the next specified step.

Table 4.5A.1-1: Procedure for IP address allocation in the U-plane

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: Step 1 below and Step 1 in Table 4.5A.1-2 describe behaviour that depends on the contents of the latest PDN CONNECTIVITY REQUEST message sent by the UE prior to this procedure. | - | - |
| - | EXCEPTION: In parallel to the event described in step 1 below the step specified in Table 4.5A.1-2 may take place. | - | - |
| 1 | If the "PDN type" in the latest PDN CONNECTIVITY REQUEST message prior to this procedure was 'IPv4' or 'IPv4v6' then, IPv4 address allocation by DHCPv4 may occur on the user plane bearer established for the default EPS bearer context activated with the latest ACTIVATE DEFAULT EPS BEARER CONTEXT message prior to this procedure. | - | - |

Table 4.5A.1-2: Procedure for IP address allocation in the U-plane, parallel behaviour

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | If the "PDN type" in the latest PDN CONNECTIVITY REQUEST message prior to this procedure was 'IPv6' or 'IPv4v6' then stateless address auto configuration occurs on the user plane bearer established for the default EPS bearer context activated with the latest ACTIVATE DEFAULT EPS BEARER CONTEXT message prior to this procedure. | - | - |

### 4.5A.2	Tracking area updating procedure

The procedure is defined in table 4.5A.2.1-1.

Table 4.5A.2.1-1: Tracking area updating procedure

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The SS transmits system information on the cell specified in the test case. | <-- | RRC: SYSTEM INFORMATION (BCCH) |
| 2 | The UE transmits an RRCConnectionRequest message on the cell specified in the test case. | --> | RRC: RRCConnectionRequest |
| 3 | SS transmits an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| 4 | The UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and a TRACKING AREA UPDATE REQUEST message is sent to update the registration of the actual tracking area. | --> | RRC: RRCConnectionSetupComplete NAS: TRACKING AREA UPDATE REQUEST |
| 5 | SS responds with TRACKING AREA UPDATE ACCEPT message. | <-- | RRC: DLInformationTransfer NAS: TRACKING AREA UPDATE ACCEPT |
| 6 | The UE transmits a TRACKING AREA UPDATE COMPLETE | --> | RRC: ULInformationTransfer NAS: TRACKING AREA UPDATE COMPLETE |
| 7 | The SS transmits an RRCConnectionRelease message to release RRC connection and move to RRC_IDLE. | <-- | RRC: RRCConnectionRelease |

NOTE 1:	The periodic tracking area updating timer T3412 is deactivated by default during the attach procedure (TS 36.508 clause 4.7.2).

NOTE 2:	The SS does not initiate authentication and NAS SECURITY MODE COMMAND are not performed (reuse of keys allocated during the attach procedure).

### 4.5A.3	Procedure for IMS signalling

The purpose of this procedure is to allow the successful completion of IMS signalling if it is initiated by the UE.

The UE may initiate IMS registration according TS 24.229 [40] clause 5.1. The procedure is applicable for UEs with IMS support (TS 36.523-2 A.4.4-1/25).

Table 4.5A.3-1: Procedure for IMS signalling in the U-plane

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1-9 | Void | - | - |
| - | EXCEPTION: Steps 10a1 to 10a2b1 describe a transaction that depends on the UE capability |  |  |
| 10a1 | IF pc_IMS then the SS starts timer Timer_1 = 10 s (Note 1) | - | - |
| - | EXCEPTION: Steps 10a2a1 to 10a2b1 describe a transaction that depends on the UE implementation | - | - |
| 10a2a1-10a2a9 | Registration procedure according TS 34.229-1 [43] subclause C.2 (steps 3-11) Note: SS cancels timer Timer_1 at step 10a2a1. | - | - |
| 10a2b1 | The SS waits for Timer_1 expiry | - | - |
| Note 1: Depending on the UE configuration there may be unpredictable delay in the start of the procedure. A guarding time of [10] sec is suggested within which the procedure is expected to start. If the timer expires then the test procedure, from which the Procedure for IMS signalling U-plane is called, shall advance to the next specified step |  |  |  |

#### 4.5A.3.1	Specific message contents

REGISTER (Step 10a2a2)

Use the default message “REGISTER” in TS 34.229-1 [43] annex A.1.1 with conditions A1, A31 and (NOT A34).

REGISTER (Step 10a2a4)

Use the default message “REGISTER” in TS 34.229-1 [43] annex A.1.1 with conditions A2, A31 and (NOT A34).

### 4.5A.3A	Procedure for IMS Signalling over UTRA

The purpose of this procedure is to allow the successful completion of IMS signalling if it is initiated by the UE.

The UE may initiate IMS registration according TS 24.229 [40] clause 5.1. The procedure is applicable for UEs with IMS support (TS 36.523-2 A.4.4-1/25).

#### 4.5A.3A.1	Initial conditions

System Simulator:

-	1 UTRA cell, default parameters.

-	The procedure shall be performed under ideal radio conditions as defined in clause 5

User Equipment:

- 	The Test UICC shall be inserted. This shall contain either ISIM and USIM applications or only a USIM application on UICC.

-	The UE is in state Registered, Idle Mode (state 3 or 7) according to TS 34.108 [5]

#### 4.5A.3A.2	Procedure

Table 4.5A.3A.2-1: Procedure for IMS Signalling over UTRA

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: Steps 1a1 to 1a2b1 describe a transaction that depends on UE capability. | - | - |
| 1a1 | IF pc_IMS then the SS starts timer Timer_1 = 10 s | - | - |
|  | EXCEPTION: Steps 1a2a1 to 1a2a15 describe a transaction that depends on the UE supporting IMS over UTRA. | - | - |
| 1a2a1 | UE transmits a RRC CONNECTION REQUEST message. | --> | RRC CONNECTION REQUEST |
| 1a2a2 | SS transmits a RRC CONNECTION SETUP message. | <-- | RRC CONNECTION SETUP |
| 1a2a3 | The UE transmits a RRC CONNECTION SETUP COMPLETE message | --> | RRC CONNECTION SETUP COMPLETE |
| 1a2a4 | The UE transmits a SERVICE REQUEST message | --> | INITIAL DIRECT TRANSFER |
| 1a2a5 | SS transmits a GMM AUTHENTICATION AND CIPHERING REQUEST message | <-- | GMM AUTHENTICATION AND CIPHERING REQUEST |
| 1a2a6 | The UE transmits a GMM AUTHENTICATION AND CIPHERING RESPONSE | --> | GMM AUTHENTICATION AND CIPHERING RESPONSE |
| 1a2a7 | SS transmits a SECURITY MODE COMMAND message | <-- | SECURITY MODE COMMAND |
| 1a2a8 | UE transmits a SECURITY MODE COMPLETE message | --> | SECURITY MODE COMPLETE |
| 1a2a9 | The UE transmits a ACTIVATE PDP CONTEXT REQUEST message | --> | ACTIVATE PDP CONTEXT REQUEST |
| 1a2a10 | The SS transmits a RADIO BEARER SETUP message | <-- | RADIO BEARER SETUP |
| 1a2a11 | The UE transmits a RADIO BEARER SETUP COMPLETE message | --> | RADIO BEARER SETUP COMPLETE |
| 1a2a13 | The SS transmits a ACTIVATE PDP CONTEXT ACCEPT message | <-- | ACTIVATE PDP CONTEXT ACCEPT |
| - | IF initiated by the UE for IP address allocation | - | - |
| 1a2a13A1a | IP address allocation in the U-plane specified in TS 34.108 [5] subclause 7.2.6 takes place performing IP address allocation in the U-plane. | - | - |
| 1a2a13A2-1a2a13A10 | Registration procedure according TS 34.229-1 [43] subclause C.2 (steps 3-11) whereby the UE is allowed to retransmit the requests in steps 4, 6, and 8 when using UDP. Note: SS cancels timer Timer_1 at step 1a2a13a1. | - | - |
| 1a2a14 | Upon completion of the U-plane signalling, the SS transmits a RRC CONNECTION RELEASE message | <-- | RRC CONNECTION RELEASE |
| 1a2a15 | The UE transmits a RRC CONNECTION RELEASE COMPLETE message | --> | RRC CONNECTION RELEASE COMPLETE |
| 1a2b1 | The SS waits for Timer_1 expiry |  |  |

#### 4.5A.3A.3	Specific message contents

Table 4.5A.3A.3-1: ACTIVATE PDP CONTEXT REQUEST (Step 1a2a9)

| Derivation Path: 24.008 Table 9.5.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Requested NSAPI |  |  |  |
| NSAPI value | ‘0101’B | NSAPI 5 |  |
| Requested LLC SAPI |  |  |  |
| LLC SAPI value | ‘0011’B | LLC SAPI 3 |  |
| Requested PDP address |  |  |  |
| PDP type organisation | ‘0001’ | IETF allocated address |  |
| PDP type number | Any value between ‘00100001’B, ‘01010111’B, ‘10001101’B | The allowed values are respectively IPv4, IPv6, IPv4v6 and "unused but interpreted as Ipv4 by the network" |  |
| Address information | Not present |  |  |
| Access point name | Not present or any allowed value |  |  |
| Protocol configuration options | Any allowed Value | The SS shall remember this IE and its contents because this affects subsequent SS behaviour, e.g. coding of ACTIVATE PDP CONTEXT ACCEPT |  |

Table 4.5A.3A.3-2: ACTIVATE PDP CONTEXT ACCEPT (Step 1a2a13)

| Derivation Path: 24.008 Table 9.5.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Negotiated LLC SAPI |  |  |  |
| LLC SAPI value | Same as requested in ACTIVATE PDP CONTEXT REQUEST |  |  |
| PDP address |  |  |  |
| PDP type number | ‘00100001’B | IPv4 | IPv4 |
| PDN address information | IPv4 address | The SS provides a valid IPv4 address | NOT IPv4-DHCP |
|  | 0.0.0.0 | DHCPv4 is to be used to allocate the IPv4 address | IPv4-DHCP |
| PDP type number | ‘01010111’B | IPv6 | IPv6 |
| PDN address information | IPv6 address | The SS provides a valid IPv6 address |  |
| PDP type number | ‘10001101’B | IPv4v6 | IPv4v6 |
| PDN address information (Octets 5 to 8) | IPv4 address | The SS provides a valid IPv4 address | NOT IPv4-DHCP |
|  | 0.0.0.0 | DHCPv4 is to be used to allocate the IPv4 address | IPv4-DHCP |
| PDN address information (Octets 9 to 24) | IPv6 address | The SS provides a valid IPv6 address | IPv6 |
| Protocol configuration options |  |  |  |
| Configuration protocol | ‘000’B |  |  |
| Container ID 1 | ‘0001’H |  | P-CSCF IPv6 |
| Length of container ID 1 contents |  | Length value determined by the TTCN implementation |  |
| Container ID 1 contents | IPv6 address | P-CSCF IPv6 Address |  |
| Container ID 2 | ‘000C’H |  | P-CSCF IPv4 |
| Length of container ID 2 contents |  | Length value determined by the TTCN implementation |  |
| Container ID 2 contents | IPv4 address | P-CSCF IPv4 Address |  |
| Container ID n | ‘0003’H | n assigned to next available number | DNS IPv6 |
| Length of container ID n contents |  | Length value determined by the TTCN implementation |  |
| Container ID n contents | IPv6 address | DNS IPv6 Address |  |
| Container ID n+1 | ‘000D’H | n assigned to next available number | DNS IPv4 |
| Length of container ID n+1 contents |  | Length value determined by the TTCN implementation |  |
| Container ID n+1 contents | IPv4 address | DNS IPv4 Address |  |

| Condition | Explanation |
| --- | --- |
| IPv4 | If the ACTIVATE PDP CONTEXT REQUEST message, contains the PDN type as Ipv4 |
| IPv6 | If the ACTIVATE PDP CONTEXT REQUEST message, contains the PDN type as Ipv6 |
| IPv4v6 | If the ACTIVATE PDP CONTEXT REQUEST message, contains the PDN type as Ipv4v6 |
| IPv4-DHCP | If the ACTIVATE PDP CONTEXT REQUEST message sent prior to this message, the IE Protocol configuration options contains a configuration protocol option "IPv4 address allocation via DHCPv4", length of contents = 0). Note 1: This condition is used in conjunction with IPv4 or IPv4v6 as indicated in the "PDN address row" just above. |
| P-CSCF IPv6 | If the ACTIVATE PDP CONTEXT REQUEST sent prior to this message, the Protocol configuration options and the additional parameter list was included with a “P-CSCF IPv6 Address Request”. Note 1: This condition is only applicable for UEs with IMS support (TS 36.523-2 A.4.4-1/25). |
| P-CSCF IPv4 | If in the ACTIVATE PDP CONTEXT REQUEST message sent prior to this message, the Protocol configuration options and the additional parameter list was included with a “P-CSCF IPv4 Address Request” Note 1: This condition is only applicable for UEs with IMS support (TS 36.523-2 A.4.4-1/25). |
| DNS IPv6 | If the ACTIVATE PDP CONTEXT REQUEST message sent prior to this message, the Protocol configuration options and the additional parameter list was included with a “DNS IPv6 Address Request”. |
| DNS IPv4 | If the ACTIVATE PDP CONTEXT REQUEST message sent prior to this message, the Protocol configuration options and the additional parameter list was included with a “DNS IPv4 Address Request”. |

### 4.5A.3B	Procedure for preventing IMS Signalling over GERAN

The purpose of this procedure is to prevent IMS signalling if it is initiated by the UE.

The UE may initiate IMS registration according TS 24.229 [40] clause 5.1. The procedure is applicable for UEs with IMS support (TS 36.523-2 A.4.4-1/25).

#### 4.5A.3B.1	Initial conditions

System Simulator:

-	1 GERAN cell, default parameters.

-	The procedure shall be performed under ideal radio conditions as defined in clause 5

User Equipment:

- 	The Test UICC shall be inserted. This shall contain either ISIM and USIM applications or only a USIM application on UICC.

-	The UE is GPRS attached according to TS 51.010 [25]

#### 4.5A.3B.2	Procedure

Table 4.5A.3B.2-1: Procedure for IMS Signalling over GERAN

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: Steps 1a1 to 1a2b1 describe a transaction that depends on UE capability. | - | - |
| 1a1 | IF pc_IMS then the SS starts timer Timer_1 = 10 s | - | - |
|  | EXCEPTION: Steps 1a2a1 to 1a2a4 describe a transaction that depends on the UE supporting IMS over GERAN | - | - |
| 1a2a1 | UE transmits CHANNEL REQUEST | --> | CHANNEL REQUEST |
| 1a2a2 | SS sends IMMEDIATE ASSIGNMENT | <-- | IMMEDIATE ASSIGNMENT |
| 1a2a3 | The UE transmits a ACTIVATE PDP CONTEXT REQUEST message | --> | ACTIVATE PDP CONTEXT REQUEST |
| 1a2a4 | The SS transmits a ACTIVATE PDP CONTEXT REJECT message | <-- | ACTIVATE PDP CONTEXT REJECT |
| 1a2a5 | The SS waits for 5 seconds | - | - |
| 1a2b1 | The SS waits for Timer_1 expiry |  |  |

#### 4.5A.3B.3	Specific message contents

Table 4.5A.3B.3-1: ACTIVATE PDP CONTEXT REQUEST (Step 1a2a3)

| Derivation Path: 24.008 Table 9.5.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Requested NSAPI |  |  |  |
| NSAPI value | ‘0101’B | NSAPI 5 |  |
| Requested LLC SAPI |  |  |  |
| LLC SAPI value | ‘0011’B | LLC SAPI 3 |  |
| Requested PDP address |  |  |  |
| PDP type organisation | ‘0001’ | IETF allocated address |  |
| PDP type number | Any allowed value | The allowed values are respectively IPv4, IPv6, IPv4v6 and "unused but interpreted as Ipv4 by the network" |  |
| Address information | Not present |  |  |
| Access point name | Not present or any allowed value |  |  |
| Protocol configuration options | Any allowed Value | The SS shall remember this IE and its contents because this affects subsequent SS behaviour, e.g. coding of ACTIVATE PDP CONTEXT ACCEPT |  |

Table 4.5A.3B.3-2: ACTIVATE PDP CONTEXT REJECT (Step 1a2a4)

| Derivation Path: 24.008 Table 9.5.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Protocol discriminator | '1010'B |  |  |
| Transaction identifier | Same as requested in ACTIVATE PDP CONTEXT REQUEST |  |  |
| Activate PDP context reject message identity | ‘01000011’B |  |  |
| SM cause | '42'O | #66 Requested APN not supported in current RAT and PLMN combination |  |
| Protocol configuration options | omit |  |  |
| Back-off timer value | omit |  |  |
| Re-attempt indicator | omit |  |  |

### 4.5A.4	Generic Test Procedure for IMS Emergency call establishment in EUTRA: Normal Service

#### 4.5A.4.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall be in Registered, Idle Mode state (State 2).

#### 4.5A.4.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.4.3	Procedure

The establishment of IMS emergency call is assumed to always be mobile originated.

Table 4.5A.4.3-1: EUTRA/EPS signalling for IMS Emergency Call

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Make the UE attempt an IMS emergency call | - | - |
| 2 | The UE transmits an RRCConnectionRequest message with 'establishmentCause' set to 'emergency'. | --> | RRCConnectionRequest |
| 3 | SS transmit an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| 4 | The UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the session management procedure by including the SERVICE REQUEST message. | --> | RRC: RRCConnectionSetupComplete NAS: SERVICE REQUEST |
| 5 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |
| 6 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 7 | The SS configures a new data radio bearer, associated with the default EPS bearer context. The RRCConnectionReconfiguration message is using condition SRB2-DRB(1, 0). The DRB associated with default EPS bearer context obtained during the attach procedure is established | <-- | RRC: RRCConnectionReconfiguration |
| 8 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer context. | --> | RRC: RRCConnectionReconfigurationComplete |
| 9 | The UE transmits a PDN CONNECTIVITY REQUEST message to request an additional PDN, with 'Request type' set to 'emergency ('0100'B )'. | --> | PDN CONNECTIVITY REQUEST |
| 10 | The SS configures a new data radio bearer, associated with the default EPS bearer context. RRCConnectionReconfiguration message contains the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message. EPS bearer context #2 (QCI 5) according to table 6.6.1-1: Reference default EPS bearer context is used. Note: The APN is set to the test APN defined for emergency bearer services. | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST |
| - | EXCEPTION: In parallel to the events described in steps 11 to 15 below, the behaviour in table 4.5A.4.3-2 occurs. (Optional IP address allocation followed by IMS emergency registration and IMS emergency speech call establishment) | - | - |
| 11 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer for emergency call. | --> | RRC: RRCConnectionReconfigurationComplete |
| 12 | The UE transmits an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer NAS:ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |
| 13 | The SS configures a new RLC-UM data radio bearer, associated with the dedicated EPS bearer context. RRCConnectionReconfiguration message contains the ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message. EPS bearer context #4 (QCI 1) according to table 6.6.2-1: Reference dedicated EPS bearer contexts is used. Note: the same PDN address is applicable because the linked EPS bearer ID refers to the default EBC allocated in step 10 | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST |
| 14 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer for emergency IMS signalling. | --> | RRC: RRCConnectionReconfigurationComplete |
| 15 | The UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer NAS:ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |

Table 4.5A.4.3-2: Parallel behaviour

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: Step 1 describes behaviour that depends on the UE behaviour. | - | - |
| 1 | If initiated by the UE the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane. | - | - |
| 2-5 | Steps 1-4 expected sequence defined in annex C.20 of TS 34.229-1 [35]. Emergency registration procedure is performed. | - | - |
| 6-10 | Steps 1-5 defined in annex C.22 of TS 34.229-1 [35]. IMS Emergency call for EPS is established. | - | - |

#### 4.5A.4.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7 with the exceptions below.

Table 4.5A.4.4-1: Message ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST (step 10, Table 4.5A.4.3-1)

| Derivation path: Table 4.7.3-6 and table 4.6.1-8 with condition AM-DRB-ADD(2) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Access point name | sos | APN value as recommended by IR.88 clause 6.4 [56] |  |
| EPS QoS | According to reference default EPS bearer context #2 – in table 6.6.1-1 | SS defines an additional dedicated EPS QoS |  |

Table 4.5A.4.4-3: Message ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST (step 13, Table 4.5A.4.3-1)

| Derivation path: Table 4.7.3-3 and table 4.6.1-8 with condition UM-DRB-ADD(3) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Linked EPS bearer identity | Default EBId-2 (same value like in table 4.5A.4.4-1) |  |  |
| EPS QoS | According to reference dedicated EPS bearer context #1 - in table 6.6.2-1 | SS defines an additional dedicated EPS QoS |  |
| TFT | According to reference dedicated EPS bearer context #1 - in table 6.6.2-1 |  |  |

Table 4.5A.4.4-4: PDN CONNECTIVITY REQUEST (step 9)

| Derivation Path: Table 4.7.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Access point name |  | Not present or any value |  |  |

### 4.5A.5	Generic Test Procedure for IMS Emergency call establishment in EUTRA: Limited Service

#### 4.5A.5.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE is switched on and camped on the cell in EMM-DEREGISTERED.LIMITED-SERVICE state.

#### 4.5A.5.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.5.3	Procedure

The establishment of IMS emergency call is assumed to always be mobile originated.

Table 4.5A.5.3-1: EUTRA/EPS signalling for IMS Emergency Call in limited service

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Make the UE attempt an IMS emergency call | - | - |
| 2 | The UE transmits an RRCConnectionRequest message with 'establishmentCause' set to 'emergency'. | --> | RRCConnectionRequest |
| 3 | SS transmits an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| 4 | The UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the Attach procedure by including the ATTACH REQUEST message, EPS attach type set to “EPS emergency attach" ('0110'B). The PDN CONNECTIVITY REQUEST message is piggybacked in ATTACH REQUEST, with 'Request type' set to 'emergency ('0100'B)'. | --> | RRC: RRCConnectionSetupComplete NAS: ATTACH REQUEST NAS: PDN CONNECTIVITY REQUEST |
| - | EXCEPTION: Steps 5a1 to 5a2b1 describe behaviour that depends on the UE ID sent in the ATTACH REQUEST message in step 4 and take place only if the UE does not send IMEI. | - | - |
| 5a1 | The SS transmits an AUTHENTICATION REQUEST message to initiate the EPS authentication and AKA procedure. | <-- | RRC: DLInformationTransfer NAS: AUTHENTICATION REQUEST |
| - | EXCEPTION: Steps 5a2a1 to 5a2b1 describe behaviour that depends on the UE state; the "lower case letter" identifies a step sequence that takes place if the UE is in a particular state. | - | - |
| 5a2a1 | UE transmits an AUTHENTICATION RESPONSE message and establishes mutual authentication. | --> | RRC: ULInformationTransfer NAS: AUTHENTICATION RESPONSE |
| 5a2b1 | UE transmits an AUTHENTICATION FAILURE message with EMM cause #20 "MAC failure" or EMM cause #21 "synch failure". (Note 2) | --> | RRC: ULInformationTransfer NAS: AUTHENTICATION FAILURE |
| 6a1 | Void | - | - |
| 6b1 | Void | - | - |
| 7 | The SS transmits a NAS SECURITY MODE COMMAND message to activate NAS security selecting. Note: If UE has sent Authentication Failure in step 5a2b16b1 or the behaviour described in steps 5a1 to 5a2b1 does not take place, KSI value is set to "000" and EIA0 (NULL integrity), and EEA0 (NULL ciphering) algorithms are used. | <-- | RRC: DLInformationTransfer NAS: SECURITY MODE COMMAND |
| 8 | The UE transmits a NAS SECURITY MODE COMPLETE message and establishes the initial security configuration. | --> | RRC: ULInformationTransfer NAS: SECURITY MODE COMPLETE |
| - | EXCEPTION: Steps 9a1 to 9a2 describe behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that take place if the UE has ESM information which needs to be transferred. | - | - |
| 9a1 | IF the UE sets the ESM information transfer flag in the last PDN CONNECTIVITY REQUEST message THEN the SS transmits an ESM INFORMATION REQUEST message to initiate exchange of protocol configuration options and/or APN. | <-- | RRC: DLInformationTransfer NAS: ESM INFORMATION REQUEST |
| 9a2 | The UE transmits an ESM INFORMATION RESPONSE message to transfer protocol configuration options and/or APN. | --> | RRC: ULInformationTransfer NAS: ESM INFORMATION RESPONSE |
| 10 | The SS transmits a SecurityModeCommand message to activate AS security. Note: If UE has sent Authentication Failure in step 5a2b1, or the behaviour described in steps 5a1 to 5a2b1 does not take place, then, eia0 (NULL integrity), and eea0 (NULL ciphering) algorithms are used. | <-- | RRC: SecurityModeCommand |
| 11 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 12 | The SS transmits a UECapabilityEnquiry message to initiate the UE radio access capability transfer procedure. | <-- | RRC: UECapabilityEnquiry |
| 13 | The UE transmits a UECapabilityInformation message to transfer UE radio access capability. | --> | RRC: UECapabilityInformation |
| 14 | The SS transmits an RRCConnectionReconfiguration message to establish the default bearer with condition SRB2-DRB(1, 0). The DRB associated with default EPS bearer context #2 (QCI 5) according to table 6.6.1-1: Reference default EPS bearer contexts, obtained during the attach procedure, is established. This message includes the ATTACH ACCEPT message with EPS attach result set to “EPS only" ('001'B). The ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message is piggybacked in ATTACH ACCEPT. Note: The APN is set to the test APN defined for emergency bearer services | <-- | RRC: RRCConnectionReconfiguration NAS: ATTACH ACCEPT NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST |
| - | EXCEPTION: In parallel to the events described in steps 15 to 19 below, the behaviour in table 4.5A.5.3-2 occurs. (Optional IP address allocation followed by IMS emergency speech call establishment) | - | - |
| 15 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of default bearer. | --> | RRC: RRCConnectionReconfigurationComplete |
| 16 | This message includes the ATTACH COMPLETE message. The ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message is piggybacked in ATTACH COMPLETE. | --> | RRC: ULInformationTransfer NAS: ATTACH COMPLETE NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |
| 17 | The SS configures a new RLC-UM data radio bearer, associated with the dedicated EPS bearer context. RRCConnectionReconfiguration message contains the ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message. EPS bearer context #4 (QCI 1) according to table 6.6.2-1: Reference dedicated EPS bearer contexts is used. Note: the same PDN address is applicable because the linked EPS bearer ID refers to the default EBC allocated in step 10 | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST |
| 18 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer for emergency IMS signalling. | --> | RRC: RRCConnectionReconfigurationComplete |
| 19 | The UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer NAS:ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |
| Note 1: Void Note 2: EMM cause #26 "non-EPS authentication unacceptable" is not expected to happen as SS will not set the "separation bit" in the AMF field of AUTN as 0 |  |  |  |

Table 4.5A.5.3-2: Parallel behaviour

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: Step 1 describes behaviour that depends on the UE behaviour. | - | - |
| 1 | If initiated by the UE the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane. | - | - |
| 2-6 | Steps 1-5 defined in annex C.22 of TS 34.229-1 [35]. IMS Emergency call for EPS is established. | - | - |

#### 4.5A.5.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7 with the exceptions below.

Table 4.5A.5.4-1: SECURITY MODE COMMAND (step 7, Table 4.5A.5.3-1)

| Derivation Path: Table 4.7.2-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Type of integrity protection algorithm |  | Set according to PIXIT parameter for default integrity protection algorithm |  | NOT NullAlgorithmUsed |
|  |  | EIA0 |  | NullAlgorithmUsed |
| Type of ciphering algorithm |  | Set according to PIXIT parameter for default ciphering algorithm |  | NOT NullAlgorithmUsed |
|  |  | EEA0 |  | NullAlgorithmUsed |
| NAS key set identifier |  |  |  |  |
| NAS key set identifier |  | The valid NAS key set identifier. |  | NOT NullAlgorithmUsed |
|  |  | '000'B |  | NullAlgorithmUsed |
| TSC |  | '0'B | native security context (for KSIASME) |  |
| Spare half octet |  | '0000'B |  |  |

| Condition | Explanation |
| --- | --- |
| NullAlgorithmUsed | UE has sent Authentication Failure in step 5ab1 (Table 4.5A.5.3-1), or the behaviour described in steps 5a1 to 5a2b1 does not take place (Table 4.5A.5.3-1). |

Table 4.5A.5.4-2: SecurityModeCommand (step 10, Table 4.5A.5.3-1)

| Derivation Path: Table 4.6.1-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SecurityModeCommand ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| securityModeCommand-r8 SEQUENCE { |  |  |  |  |
| securityConfigSMC |  | SecurityConfigSMC-LimitedService | Table 4.5A.5.4-3 |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 4.5A.5.4-3: SecurityConfigSMC-LimitedService (Table 4.5A.5.4-2)

| Derivation Path: Table 4.6.4-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SecurityConfigSMC- LimitedService ::= SEQUENCE { |  |  |  |  |
| securityAlgorithmConfig SEQUENCE { |  |  |  |  |
| cipheringAlgorithm |  | Set according to PIXIT parameter for default ciphering algorithm |  | NOT NullAlgorithmUsed |
|  |  | eea0 |  | NullAlgorithmUsed |
| integrityProtAlgorithm |  | Set according to PIXIT parameter for default integrity protection algorithm |  | NOT NullAlgorithmUsed |
|  |  | eia0 |  | NullAlgorithmUsed |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| NullAlgorithmUsed | UE has sent Authentication Failure in step 5a2b1 (Table 4.5A.5.3-1), or the behaviour described in steps 5a1 to 5a2b1 (Table 4.5A.5.3-1) does not take place. |

Table 4.5A.5.4-4: Message ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST (step 17, Table 4.5A.5.3-1)

| Derivation path: Table 4.7.3-3 and table 4.6.1-8 with condition UM-DRB-ADD(3) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Linked EPS bearer identity | Default EBId-2 (same value as in table 4.5A.5.4-4) |  |  |
| EPS QoS | See Reference dedicated EPS bearer context #4 in table 6.6.2-1 |  |  |
| TFT | See Reference dedicated EPS bearer context #4 in table 6.6.2-1 |  |  |

Table 4.5A.5.4-5: Message ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST (step 14, Table 4.5A.5.3-1)

| Derivation path: Table 4.7.3-6 and table 4.6.1-8 with condition AM-DRB-ADD(2) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Access point name | sos | APN value as recommended by IR.88 clause 6.4 [56] |  |
| EPS QoS | According to reference default EPS bearer context #2 – in table 6.6.1-1 | SS defines an additional dedicated EPS bearer |  |

Table 4.5A.5.4-6: Message ESM INFORMATION RESPONSE (step 9a2, Table 4.5A.5.3-1)

| Derivation Path: Table 4.7.3-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Access point name |  | Not present or any value |  |  |

### 4.5A.6	Generic Test Procedure for IMS MO speech call establishment in E-UTRA

#### 4.5A.6.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall be in Registered, Idle Mode state (State 2).

#### 4.5A.6.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.6.3	Procedure

Table 4.5A.6.3-1: EUTRA/EPS signalling for IMS MO speech call

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Make the UE attempt an IMS speech call | - | - |
| 2 | The UE transmits an RRCConnectionRequest message with ' establishmentCause' set to ' mo-Data '. | --> | RRCConnectionRequest |
| 3 | SS transmit an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| 4 | The UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the session management procedure by including the SERVICE REQUEST message. | --> | RRC: RRCConnectionSetupComplete NAS: SERVICE REQUEST |
| 5 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |
| 6 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 7 | The SS configures a new data radio bearer, associated with the default EPS bearer context. The RRCConnectionReconfiguration message is using condition SRB2-DRB(1, 0). The DRB associated with default EPS bearer context obtained during the attach procedure is established | <-- | RRC: RRCConnectionReconfiguration |
| - | EXCEPTION: In parallel to the events described in steps 8 below, the behaviour in table 4.5A.6.3-2 occurs. (IMS MTSI MO speech call establishment) | - | - |
| 8 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer context. | --> | RRC: RRCConnectionReconfigurationComplete |
| - | EXCEPTION: Steps 9a1 to 11b1 describe behaviour that depends on the UE category. | - | - |
| 9a1-11a1 | IF not pc_ue_CategoryDL_M1 THEN steps 3-4 from the expected sequence defined in annex C.21of TS 34.229-1 [35]. MTSI MO speech call for EPS take place. | - | - |
| 9b1-11b1 | IF pc_ue_CategoryDL_M1 THEN steps 3-4 from the expected sequence defined in annex C.21d of TS 34.229-1 [35]. MTSI MO speech call for EPS / UE category M1 take place. | - | - |
| 12 | The SS configures a new RLC-UM data radio bearer with condition DRB (0,1), associated with the dedicated EPS bearer context. RRCConnectionReconfiguration message contains the ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message. EPS bearer context #4 (QCI 1) according to table 6.6.2-1: Reference dedicated EPS bearer contexts. | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST |
| - | EXCEPTION: In parallel to the events described in steps 13-14 below, the behaviour in table 4.5A.6.3-3 occurs. (IMS MTSI MO speech call establishment) | - | - |
| 13 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the dedicated EPS bearer. | --> | RRC: RRCConnectionReconfigurationComplete |
| 14 | The UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer NAS:ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |

Table 4.5A.6.3-2: Parallel behaviour

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | . EXCEPTION: Steps 1a1 to 1b1 describe behaviour that depends on the UE category. |  |  |
| 1a1 | IF not pc_ue_CategoryDL_M1 THEN THEN the step 2 expected sequence defined in annex C.21 of TS 34.229-1 [35]. UE sends INVITE take place. | - | - |
| 1b1 | IF pc_ue_CategoryDL_M1 THEN THEN Step 2 from the expected sequence defined in annex C.21d of TS 34.229-1 [35]. UE sends INVITE take place. | - | - |

Table 4.5A.6.3-3: Parallel behaviour

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | . EXCEPTION: Steps 1a1 to 8b1 describe behaviour that depends on the UE category. |  |  |
| 1a1-8a1 | IF not pc_ue_CategoryDL_M1 THEN HEN the steps from the 5-13 expected sequence defined in annex C.21 of TS 34.229-1 [35]. MTSI MO speech call for EPS take place. | - | - |
| 1b1-8b1 | IF pc_ue_CategoryDL_M1 THEN the steps 5-13 from the expected sequence defined in annex C.21d of TS 34.229-1 [35]. MTSI MO speech call for EPS / UE category M1 take place. | - | - |

#### 4.5A.6.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7.

### 4.5A.7	Generic Test Procedure for IMS MT Speech call establishment in E-UTRA

#### 4.5A.7.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall be in Registered, Idle Mode state (State 2).

#### 4.5A.7.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.7.3	Procedure

Table 4.5A.7.3-1: EUTRA/EPS signalling for IMS MT speech call

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | SS sends a Paging message to the UE on the appropriate paging block, and including the UE identity in one entry of the IE pagingRecordLists. | <-- | RRC: Paging (PCCH) |
| 2 | The UE transmits an RRCConnectionRequest message with ' establishmentCause' set to 'mt-Access'. | --> | RRCConnectionRequest |
| 3 | SS transmit an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| 4 | The UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the session management procedure by including the SERVICE REQUEST message. | --> | RRC: RRCConnectionSetupComplete NAS: SERVICE REQUEST |
| 5 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |
| 6 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 7 | The SS configures a new data radio bearer, associated with the default EPS bearer context. The RRCConnectionReconfiguration message is using condition SRB2-DRB(1, 0). The DRB associated with default EPS bearer context obtained during the attach procedure is established | <-- | RRC: RRCConnectionReconfiguration |
| 8 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer context. | --> | RRC: RRCConnectionReconfigurationComplete |
| - | EXCEPTION: Steps 9a1 to 12b3 describe behaviour that depends on the UE category. |  |  |
| 9a1-12a3 | IF not pc_ue_CategoryDL_M1 THEN THEN the steps 1-6 from the expected sequence defined in annex C.11 of TS 34.229-1 [35]. MTSI MT speech call take place. | - | - |
| 9b1-12b3 | IF pc_ue_CategoryDL_M1 THEN steps 1-6 from the expected sequence defined in annex C.11d of TS 34.229-1 [35]. MTSI MT speech call / UE category M1 take place. | - | - |
| 13 | The SS configures a new RLC-UM data radio bearer with condition DRB (0,1), associated with the dedicated EPS bearer context. RRCConnectionReconfiguration message contains the ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message. | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST |
| 14 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the dedicated EPS bearer. | --> | RRC: RRCConnectionReconfigurationComplete |
| 15 | The UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer NAS:ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |
| - | EXCEPTION: Steps 18a1 to 22b1 describe behaviour that depends on the UE category. |  |  |
| 16a1-17 | Void | - | - |
| 18a1-22a1 | IF not pc_ue_CategoryDL_M1 THEN steps 7-11 from the expected sequence defined in annex C.11 of TS 34.229-1 [35] MTSI MT speech call take place. | - | - |
| 18b1-22b1 | IF pc_ue_CategoryDL_M1 THEN the steps 7-11 from the expected sequence defined in annex C.11d of TS 34.229-1 [35] MTSI MT speech call / UE category M1 take place. | - | - |
| - | EXCEPTION: Steps 23Aa1 to 23Ab1 describe behaviour that depends on the UE category. |  |  |
| 23Aa1 | IF not pc_ue_CategoryDL_M1 THEN step 11A from the expected sequence defined in annex C.11 of TS 34.229-1 [35] MTSI MT speech call take place. | - | - |
| 23Ab1 | IF pc_ue_CategoryDL_M1 THEN step 11A from the expected sequence defined in annex C.11d of TS 34.229-1 [35] MTSI MT speech call / UE category M1 take place. | - | - |
| - | EXCEPTION: Steps 23a1 to 24b1 describe behaviour that depends on the UE category. |  |  |
| 23a1-24a1 | IF not pc_ue_CategoryDL_M1 THEN steps 12-13 expected sequence defined in annex C.11 of TS 34.229-1 [35] MTSI MT speech call take place. | - | - |
| 23b1-24b1 | IF pc_ue_CategoryDL_M1 THEN steps 12-13 from the expected sequence defined in annex C.11d of TS 34.229-1 [35] MTSI MT speech call / UE category M1 take place. | - | - |
| - | EXCEPTION: Steps 25a1 to 26b1 describe behaviour that depends on the UE category. |  |  |
| 25a1-26a2 | IF not pc_ue_CategoryDL_M1 THEN steps 14-15 from the expected sequence defined in annex C.11 of TS 34.229-1 [35] MTSI MT speech call take place. | - | - |
| 25b1-26b1 | IF pc_ue_CategoryDL_M1 THEN steps 14-15 from the expected sequence defined in annex C.11d of TS 34.229-1 [35] MTSI MT speech call / UE category M1 take place. | - | - |

#### 4.5A.7.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7.

### 4.5A.8	Generic Test Procedure for IMS MO video call establishment in E-UTRA

#### 4.5A.8.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall be in Registered, Idle Mode state (State 2).

#### 4.5A.8.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.8.3	Procedure

Table 4.5A.8.3-1: EUTRA/EPS signalling for IMS MO video call

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Make the UE attempt an IMS video call | - | - |
| 2 | The UE transmits an RRCConnectionRequest message with ' establishmentCause' set to ' mo-Data '. | --> | RRCConnectionRequest |
| 3 | SS transmit an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| 4 | The UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the session management procedure by including the SERVICE REQUEST message. | --> | RRC: RRCConnectionSetupComplete NAS: SERVICE REQUEST |
| 5 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |
| 6 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 7 | The SS configures a new data radio bearer, associated with the default EPS bearer context. The RRCConnectionReconfiguration message is using condition SRB2-DRB(1, 0). The DRB associated with default EPS bearer context obtained during the attach procedure is established | <-- | RRC: RRCConnectionReconfiguration |
| - | EXCEPTION: In parallel to the events described in steps 8 below, the behaviour in table 4.5A.8.3-2 occurs. (IMS MTSI MO video call establishment) | - | - |
| 8 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer context. | --> | RRC: RRCConnectionReconfigurationComplete |
| 9-11 | Steps 3-4 expected sequence defined in annex C.25 of TS 34.229-1 [35]. MTSI MO video call for EPS. | - | - |
| 12 | The SS configures two new RLC-UM data radio bearers with condition DRB (0,2), associated with the dedicated EPS bearer context. RRCConnectionReconfiguration message contains one ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message per additional data radio bearer. One EPS bearer context #4 (QCI 1) and one EPS bearer context #3 (QCI 2) according table 6.6.2-1: Reference dedicated EPS bearer contexts. | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST |
| - | EXCEPTION: In parallel to the events described in steps 13-14 below, the behaviour in table 4.5A.8.3-3 occurs. (IMS MTSI MO video call establishment). | - | - |
| 13 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the dedicated EPS bearer. EPS bearer context #4 (QCI 1) according table to 6.6.2-1: Reference dedicated EPS bearer contexts. | --> | RRC: RRCConnectionReconfigurationComplete |
| 14 | The UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message for the first bearer. | --> | RRC: ULInformationTransfer NAS:ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |
| 15 | The UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message for the second bearer. | --> | RRC: ULInformationTransfer NAS:ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |

Table 4.5A.8.3-2: Parallel behaviour

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Step 2 expected sequence defined in annex C.25 of TS 34.229-1 [35]. UE sends INVITE. | - | - |

Table 4.5A.8.3-3: Parallel behaviour

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1-8 | Steps 5-13 expected sequence defined in annex C.25 of TS 34.229-1 [35]. MTSI MO video call for EPS. | - | - |

#### 4.5A.8.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7.

### 4.5A.9	Generic Test Procedure for IMS MT video call establishment in E-UTRA

#### 4.5A.9.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall be in Registered, Idle Mode state (State 2).

#### 4.5A.9.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.9.3	Procedure

Table 4.5A.9.3-1: EUTRA/EPS signalling for IMS MT video call

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | SS sends a Paging message to the UE on the appropriate paging block, and including the UE identity in one entry of the IE pagingRecordLists. | <-- | RRC: Paging (PCCH) |
| 2 | The UE transmits an RRCConnectionRequest message with ' establishmentCause' set to 'mt-Access'. | --> | RRCConnectionRequest |
| 3 | SS transmit an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| 4 | The UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the session management procedure by including the SERVICE REQUEST message. | --> | RRC: RRCConnectionSetupComplete NAS: SERVICE REQUEST |
| 5 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |
| 6 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 7 | The SS configures a new data radio bearer, associated with the default EPS bearer context. The RRCConnectionReconfiguration message is using condition SRB2-DRB(1, 0). The DRB associated with default EPS bearer context obtained during the attach procedure is established | <-- | RRC: RRCConnectionReconfiguration |
| 8 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer context. | --> | RRC: RRCConnectionReconfigurationComplete |
| 9-12b | Steps 1-6 expected sequence defined in annex C.26 of TS 34.229-1 [35]. MTSI MT video call. | - | - |
| 13 | The SS configures a new RLC-UM data radio bearer with condition DRB (0,2), associated with the dedicated EPS bearer context. RRCConnectionReconfiguration message contains one ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message per additional data radio bearer. One EPS bearer context #4 (QCI 1) and one EPS bearer context #3 (QCI 2) according table 6.6.2-1: Reference dedicated EPS bearer contexts. | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST |
| 14 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the dedicated EPS bearer. | --> | RRC: RRCConnectionReconfigurationComplete |
| 15 | The UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message for the first bearer. | --> | RRC: ULInformationTransfer NAS:ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |
| 16 | The UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message for the second bearer. | --> | RRC: ULInformationTransfer NAS:ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |
| 17-18 | Void | - | - |
| 19-27 | Steps 7-15 expected sequence defined in annex C.26 of TS 34.229-1 [35]. MTSI MT video call for EPS. | - | - |

#### 4.5A.9.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7.

### 4.5A.10	Generic Test Procedure for IMS MO speech and aSRVCC in E-UTRA

#### 4.5A.10.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall be in Registered, Idle Mode state (State 2).

#### 4.5A.10.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.10.3	Procedure

Table 4.5A.10.3-1: EUTRA/EPS signalling for IMS MO speech to alerting state

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Make the UE attempt an IMS speech call | - | - |
| 2 | The UE transmits an RRCConnectionRequest message with ' establishmentCause' set to ' mo-Data '. | --> | RRCConnectionRequest |
| 3 | SS transmit an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| 4 | The UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the session management procedure by including the SERVICE REQUEST message. | --> | RRC: RRCConnectionSetupComplete NAS: SERVICE REQUEST |
| 5 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |
| 6 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 7 | The SS configures a new data radio bearer, associated with the default EPS bearer context. The RRCConnectionReconfiguration message is using condition SRB2-DRB(1, 0). The DRB associated with default EPS bearer context obtained during the attach procedure is established | <-- | RRC: RRCConnectionReconfiguration |
| - | EXCEPTION: In parallel to the events described in steps 8 below, the behaviour in table 4.5A.10.3-2 occurs. Initiate MTSI MO speech. | - | - |
| 8 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer context. | --> | RRC: RRCConnectionReconfigurationComplete |
| 9-11 | Steps 3-4 expected sequence defined in annex C.21 of TS 34.229-1 [35]. MTSI MO speech call for EPS. | - | - |
| 12 | The SS configures a new RLC-UM data radio bearer with condition DRB (0,1), associated with the dedicated EPS bearer context. RRCConnectionReconfiguration message contains the ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message. EPS bearer context #4 (QCI 1) according to table 6.6.2-1: Reference dedicated EPS bearer contexts. | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST |
|  | EXCEPTION: In parallel to the events described in steps 13-14 below, the behaviour in table 4.5A.10.3-3 occurs. MTSI MO speech call to alerting state. | - | - |
| 13 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the dedicated EPS bearer. | --> | RRC: RRCConnectionReconfigurationComplete |
| 14 | The UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer NAS:ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |

Table 4.5A.10.3-2: Parallel behaviour

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Step 2 expected sequence defined in annex C.21 of TS 34.229-1 [35]. UE sends INVITE. | - | - |

Table 4.5A.10.3-3: Parallel behaviour

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1-7 | Steps 5-11 expected sequence defined in annex C.21 of TS 34.229-1 [35]. MTSI MO speech call to alerting state for EPS. | - | - |

#### 4.5A.10.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7.

### 4.5A.11	Generic Test Procedure for IMS MO add video establishment in E-UTRA

#### 4.5A.11.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall execute 4.5A.6.

#### 4.5A.11.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.11.3	Procedure

Table 4.5A.11.3-1: EUTRA/EPS signalling for IMS MO add video

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Make the UE attempt add IMS video to the voice call. | - | - |
| 2-4 | Steps 2-4 expected sequence defined in expected sequence test case 17.1.4 of TS 34.229-1 [35]. MO Speech, add video remove video. | - | - |
| 5 | The SS configures a new RLC-UM data radio bearer with condition DRB (0,1), associated with the dedicated EPS bearer context. RRCConnectionReconfiguration message contains the ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message. EPS bearer context #3 (QCI 2) according to table 6.6.2-1: Reference dedicated EPS bearer contexts. | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST |
| - | EXCEPTION: In parallel to the events described in steps 6-7 below, the behaviour in table 4.5A.11.3-2 occurs. (IMS MTSI MO video call establishment). | - | - |
| 6 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the dedicated EPS bearer. EPS bearer context #3 (QCI 2) according table to 6.6.2-1: Reference dedicated EPS bearer contexts. | --> | RRC: RRCConnectionReconfigurationComplete |
| 7 | The UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message for the first bearer. | --> | RRC: ULInformationTransfer NAS:ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |

Table 4.5A.11.3-2: Parallel behaviour

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1-6 | Steps 5-10 expected sequence defined in test case 17.1.4 of TS 34.229-1 [35]. MO Speech, add video remove video. | - | - |

#### 4.5A.11.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7.

### 4.5A.12	Generic Test Procedure for IMS MT add video establishment in E-UTRA

#### 4.5A.12.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall execute 4.5A.7.

#### 4.5A.12.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.12.3	Procedure

Table 4.5A.12.3-1: EUTRA/EPS signalling for IMS MT add video

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1-3b | Steps 1-5 expected sequence defined in expected sequence test case 17.2.4 of TS 34.229-1 [35]. MT Speech, add video remove video. | - | - |
| 4 | The SS configures a new RLC-UM data radio bearer with condition DRB (0,1), associated with the dedicated EPS bearer context. RRCConnectionReconfiguration message contains the ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message. EPS bearer context #3 (QCI 2) according to table 6.6.2-1: Reference dedicated EPS bearer contexts. | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST |
| 5 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the dedicated EPS bearer. EPS bearer context #3 (QCI 2) according table to 6.6.2-1: Reference dedicated EPS bearer contexts. | --> | RRC: RRCConnectionReconfigurationComplete |
| 6 | The UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message for the first bearer. | --> | RRC: ULInformationTransfer NAS:ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |
| 7-8 | Void | - | - |
| 9-12 | Steps 6-9 expected sequence defined in test case 17.2.4 of TS 34.229-1 [35], MT Speech, add video remove video. | - | - |

#### 4.5A.12.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7.

4.5A.13	Void

### 4.5A.14	Generic Test Procedure for IMS XCAP establishment in EUTRA

#### 4.5A.14.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall be in Registered, Idle Mode state (State 2).

-	If pc_XCAP_only_APN==true then the UE is connected to the IMS PDN only

-	else the UE is connected to the IMS PDN and the Internet PDN which is used for XCAP signalling.

#### 4.5A.14.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.14.3	Procedure

Table 4.5A.14.3-1: EUTRA/EPS signalling for XCAP

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Make the UE attempt XCAP signalling. | - | - |
| 2 | The UE transmits an RRCConnectionRequest message with ' establishmentCause' set to 'mo-Data '. | --> | RRCConnectionRequest |
| 3 | SS transmit an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| 4 | The UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the session management procedure by including the SERVICE REQUEST message. | --> | RRC: RRCConnectionSetupComplete NAS: SERVICE REQUEST |
| 5 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |
| 6 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 7 | The SS configures a new data radio bearer, associated with the default EPS bearer context. The RRCConnectionReconfiguration message is using condition SRB2-DRB(1, 0) in case of pc_XCAP_only_APN==true or using condition SRB2-DRB(2, 0) in case of pc_XCAP_over_Internet_APN==true. The DRBs associated with the respective default EPS bearer context obtained during the attach procedure are established | <-- | RRC: RRCConnectionReconfiguration |
| 8 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer context. | --> | RRC: RRCConnectionReconfigurationComplete |
|  | EXCEPTION: IF pc_XCAP_only_APN THEN additional PDN connectivity shall be established as specified in TS 36.508 subclause 4.5A.16 (condition: NOT ADD_IMS) |  |  |

#### 4.5A.14.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7.

### 4.5A.15	Generic Test Procedure for EPS Bearer Deactivation

#### 4.5A.15.1	Initial conditions

The EPS bearer to be deactivated has been established before. The EPS bearer has to be specified.

#### 4.5A.15.2	Definition of system information messages

The system information messages applicable in the test case are used.

#### 4.5A.15.3	Procedure

Table 4.5A.15.3-1: Procedure for EPS Bearer Deactivation

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The SS transmits an RRCConnectionReconfiguration message to deactivate an EPS bearer. | <-- | RRC: RRCConnectionReconfiguration NAS: DEACTIVATE EPS BEARER CONTEXT REQUEST |
| - | EXCEPTION: In parallel to the event described in step2 the step specified in Table 4.5A.15.3-2 should take place. | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the deactivation of EPS bearer. | --> | RRC: RRCConnectionReconfigurationComplete |
| 3 | Void | - | - |

Table 4.5A.15.3-2: Procedure for EPS Bearer Deactivation (parallel behaviour)

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The UE transmits an ULInformationTransfer message to accept deactivation of the EPS bearer. | --> | RRC: ULInformationTransfer NAS: DEACTIVATE EPS BEARER CONTEXT ACCEPT |

#### 4.5A.15.4	Specific message contents

Table 4.5A.15.4-1: Message DEACTIVATE EPS BEARER CONTEXT REQUEST (step 1, Table 4.5A.15.3-1)

| Derivation path: Table 4.7.3-12 and table 4.6.1-8 with condition and condition NETWORK-INITIATED |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS bearer identity | EPS bearer identity | Same value as in the activation message. |  |
| ESM cause | 00100100 | regular deactivation |  |

Table 4.5A.15.4-2: Message DEACTIVATE EPS BEARER CONTEXT ACCEPT (step 2, Table 4.5A.15.3-1)

| Derivation Path:Table 4.7.3-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS bearer identity | EPS bearer identity | The same value as the value set in DEACTIVATE EPS BEARER CONTEXT REQUEST message. |  |
| Procedure transaction identity | 0 | No procedure transaction identity assigned |  |

### 4.5A.15A	Generic Test Procedure for User or Network Initiated EPS Bearer Deactivation

#### 4.5A.15A.1	Initial conditions

The EPS bearer to be deactivated has been established before. The EPS bearer has to be specified.

#### 4.5A.15A.2	Definition of system information messages

The system information messages applicable in the test case are used.

#### 4.5A.15A.3	Procedure

Table 4.5A.15A.3-1: Procedure for EPS Bearer Deactivation

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: Step 1Aa describes behaviour that depends on UE capabilities/configuration; the "lower case letter" identifies a step sequence that takes place if one of those is supported/configured. The SS shall wait up to 5s for the UE to trigger the PDN Disconnect procedure. NOTE: The specified wait period of 5s is a working assumption to facilitate test case implementation. | - | - |
| 1Aa | The UE transmits a PDN DISCONNECT REQUEST | --> | RRC: ULInformationTransfer PDN DISCONNECT REQUEST |
| 1 | The SS transmits an RRCConnectionReconfiguration message to deactivate an EPS bearer. | <-- | RRC: RRCConnectionReconfiguration NAS: DEACTIVATE EPS BEARER CONTEXT REQUEST |
| - | EXCEPTION: In parallel to the event described in step2 the step specified in Table 4.5A.15A.3-2 should take place. | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the deactivation of EPS bearer. | --> | RRC: RRCConnectionReconfigurationComplete |

Table 4.5A.15A.3-2: Procedure for EPS Bearer Deactivation (parallel behaviour)

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The UE transmits an ULInformationTransfer message to accept deactivation of the EPS bearer. | --> | RRC: ULInformationTransfer NAS: DEACTIVATE EPS BEARER CONTEXT ACCEPT |

#### 4.5A.15A.4	Specific message contents

Table 4.5A.15A.4-1: Message DEACTIVATE EPS BEARER CONTEXT REQUEST (step 1, Table 4.5A.15.3-1)

| Derivation path: Table 4.7.3-12 and table 4.6.1-8 with condition and condition NETWORK-INITIATED |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS bearer identity | EPS bearer identity | Same value as in the activation message. |  |
| ESM cause | 00100100 | regular deactivation |  |

Table 4.5A.15A.4-2: Message DEACTIVATE EPS BEARER CONTEXT ACCEPT (step 2, Table 4.5A.15.3-1)

| Derivation Path: Table 4.7.3-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS bearer identity | EPS bearer identity | The same value as the value set in DEACTIVATE EPS BEARER CONTEXT REQUEST message. |  |
| Procedure transaction identity | 0 | No procedure transaction identity assigned |  |

### 4.5A.16	Generic Test Procedure to establish additional PDN connectivity

The same assumptions and definitions apply as in clause 4.5.2and in addition:

| Condition | Explanation |
| --- | --- |
| ADD_IMS | true if this procedure is part of the initial registration (PDN2_IMS==true according to definitions in clause 4.5.2); false for all other cases |
| MobileData_OFF | Mobile data associated with Internet PDN is disabled on the DUT. It is configured via setting px_MobileDataOn (TS 36.523-3[20] Table 9.1-1) = false |

#### 4.5A.16.1	Initial conditions

System Simulator:

-	1 cell, default parameters.

-	The procedure shall be performed under ideal radio conditions as defined in clause 5

User Equipment:

- 	The UE is in Registered, RRC_CONNECTED state (State 2).

#### 4.5A.16.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.16.3	Procedure

Table 4.5A.16.3-1: Establishment of additional PDN connectivity

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: Depending upon UE configuration Step 0a1 may be performed | - | - |
| 0a1 | If IMS_Internet and MobileData_OFF then the request of connectivity to an additional PDN is performed by MMI or AT command with APN “Internet”. | - | - |
| 1 | The UE transmits a PDN CONNECTIVITY REQUEST message to request an additional PDN. | --> | RRC: ULInformationTransfer NAS: PDN CONNECTIVITY REQUEST |
| 2 | The SS configures a new data radio bearer, associated with the additional default EPS bearer context. RRCConnectionReconfiguration message contains the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message. | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST |
| 3 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of additional default bearer. | --> | RRC: RRCConnectionReconfigurationComplete |
| - | EXCEPTION: In parallel to the event described in step 4 below, if initiated by the UE the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane. | - | - |
| - | EXCEPTION: IF ADD_IMS THEN in parallel to the event described in step 4 below the generic procedure for IMS signalling in the U-plane specified in TS 36.508 subclause 4.5A.3 takes place if requested by the UE | - | - |
| 4 | The UE transmits an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |

#### 4.5A.16.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7 with the exceptions below.

Table 4.5A.16.4-1: PDN CONNECTIVITY REQUEST (step 1)

| Derivation Path: Table 4.7.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Access point name |  | Any allowed value | The UE includes a new APN. |  |

Table 4.5A.16.4-2: Message ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST (step 2)

| Derivation path: Table 4.7.3-6 with condition IMS_PDN_ConnEstab for ADD_IMS and NOT IMS_PDN_ConnEstab else and table 4.6.1-8 with condition AM-DRB-ADD(1) for ADD_IMS or with condition AM-DRB-ADD(8) else |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS bearer identity | '0101'B | arbitrary value used for PDN connectivity being maintained during the test case | ADD_IMS |
| EPS bearer identity | '1100'B | arbitrary value used for additional non-IMS PDN connectivity | NOT ADD_IMS |
| Procedure transaction identity | PTI-1 | SS re-uses the particular PTI defined by UE for this present additional PDN connectivity request procedure |  |

Table 4.5A.16.4-3: Message ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT (step 4)

| Derivation path: Table 4.7.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS bearer identity | (same value as used in step 2) |  |  |
| Procedure transaction identity | 0 | No procedure transaction identity assigned |  |

### 4.5A.17	Generic Test Procedure for user initiated release of additional PDN connectivity

#### 4.5A.17.1	Initial conditions

System Simulator:

-	1 cell, default parameters.

-	The procedure shall be performed under ideal radio conditions as defined in clause 5

User Equipment:

-	The UE shall be in Registered, Idle Mode state (State 2) with connectivity at least to two PDNs

#### 4.5A.17.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.17.3	Procedure

Table 4.5A.17.3-1: Release of additional PDN connectivity

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Cause the UE to request disconnection from the additional PDN (see Note 1) | - | - |
| 2 | The UE transmits an RRCConnectionRequest | --> | RRCConnectionRequest |
| 3 | SS transmit an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| - | EXCEPTION: Steps 4a1 to 4b1 describe behaviour which depends on earlier network indication of supported features. | - | - |
| 4a1 | IF the network indicated support of EXTENDED SERVICE REQUEST for packet services in the preceding ATTACH ACCEPT message, THEN the UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the session management procedure by including the EXTENDED SERVICE REQUEST message | --> | RRC: RRCConnectionSetupComplete NAS: EXTENDED SERVICE REQUEST |
| 4b1 | ELSE, the UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the session management procedure by including the SERVICE REQUEST message. | --> | RRC: RRCConnectionSetupComplete NAS: SERVICE REQUEST |
| 5 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |
| 6 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 7 | The SS transmits a RRCConnectionReconfiguration message to establish SRB2 and DRBs associated with all default EPS bearer contexts | <-- | RRC: RRCConnectionReconfiguration |
| 8 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of SRB2 and DRBs associated with all default EPS bearer contexts. | --> | RRC: RRCConnectionReconfigurationComplete |
| 9 | The UE transmits a PDN DISCONNECT REQUEST | --> | RRC: ULInformationTransfer PDN DISCONNECT REQUEST |
| 10 | The SS transmits a DEACTIVATE EPS BEARER CONTEXT REQUEST message included in an RRCConnectionReconfiguration message. | <-- | RRC: RRCConnectionReconfiguration NAS: DEACTIVATE EPS BEARER CONTEXT REQUEST |
| 11 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the release of the bearer. | --> | RRC: RRCConnectionReconfigurationComplete |
| 12 | The UE transmits a DEACTIVATE EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer DEACTIVATE EPS BEARER CONTEXT ACCEPT |
| 13 | The SS transmits an RRCConnectionRelease message to release RRC connection | <-- | RRC: RRCConnectionRelease |
| NOTE 1: The request to disconnect from a PDN may be performed by MMI or AT command; in any case the EPS bearer identity of the Default EPS Bearer of the PDN to be released needs to be handed over to the UE |  |  |  |

#### 4.5A.17.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7 with the exceptions below.

Table 4.5A.17.4-1: RRCConnectionReconfiguration (step 7)

Derivation path: 36.508 table 4.6.1-8 using condition SRB2-DRB(2, 0) NOTE: The bid for the AM DRBs shall be 1 and 8 instead of 1 and 2.

Table 4.5A.17.4-2: PDN DISCONNECT REQUEST (step 9)

| Derivation Path: TS 36.508 Table 4.7.3-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS bearer identity | ‘0000’ | "no EPS bearer identity assigned" |  |
| Procedure transaction identity | PTI-1 | UE assigns a particular PTI not yet used between 1 and 254 |  |
| Linked EPS bearer identity | (bearer identity as handed over at step 1) |  |  |

Table 4.5A.17.4-3: RRCConnectionReconfiguration (step 10)

Derivation path: 36.508 table 4.6.1-8 using condition DRB-REL(8)

Table 4.5A.17.4-4: DEACTIVATE EPS BEARER CONTEXT REQUEST (step 10)

| Derivation Path: TS 36.508 Table 4.7.3-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS bearer identity | (same as in Linked EPS bearer identity of step 9) |  |  |
| Procedure transaction identity | PTI-1 | SS re-uses the particular PTI defined by UE for this present PDN disconnection procedure. | UE-INITIATED |
| ESM cause | 00100100 | regular deactivation |  |

Table 4.5A.17.4-5: DEACTIVATE EPS BEARER CONTEXT ACCEPT (step 12)

| Derivation Path: TS 36.508 Table 4.7.3-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS bearer identity | (same as in DEACTIVATE EPS BEARER CONTEXT REQUEST of step 10) | The same value as the value set in DEACTIVATE EPS BEARER CONTEXT REQUEST message. |  |
| Procedure transaction identity | 0 | No procedure transaction identity assigned |  |

### 4.5A.18	Generic Test Procedure for network initiated release of additional PDN connectivity

#### 4.5A.18.1	Initial conditions

System Simulator:

-	1 cell, default parameters.

-	The procedure shall be performed under ideal radio conditions as defined in clause 5

User Equipment:

- 	The UE shall be in Registered, Idle Mode state (State 2) with connectivity at least to two PDNs

#### 4.5A.18.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.18.3	Procedure

Table 4.5A.18.3-1: Release of additional PDN connectivity

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1-7 | Steps 1 to 7 of the generic radio bearer establishment procedure (TS 36.508 4.5.3.3-1) |  |  |
| 8 | The SS transmits a RRCConnectionReconfiguration message to establish SRB2 and DRBs associated with all default EPS bearer contexts | <-- | RRC: RRCConnectionReconfiguration |
| 9 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of SRB2 and DRBs associated with all default EPS bearer contexts. | --> | RRC: RRCConnectionReconfigurationComplete |
| 10 | SS releases the PDN connectivity by transmitting a RRCConnectionReconfiguration containing a DEACTIVATE EPS BEARER CONTEXT REQUEST | <-- | RRC: RRCConnectionReconfiguration NAS: DEACTIVATE EPS BEARER CONTEXT REQUEST |
| 11 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the release of the bearer. | --> | RRC: RRCConnectionReconfigurationComplete |
| 12 | The UE transmits a DEACTIVATE EPS BEARER CONTEXT ACCEPT message. | --> | DEACTIVATE EPS BEARER CONTEXT ACCEPT |
| 13 | The SS transmits an RRCConnectionRelease message to release RRC connection | <-- | RRC: RRCConnectionRelease |

#### 4.5A.18.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7 with the exceptions below.

Table 4.5A.18.4-1: RRCConnectionReconfiguration (step 8)

Derivation path: 36.508 table 4.6.1-8 using condition SRB2-DRB(2, 0)

NOTE: The bid for the AM DRBs shall be 1 and 8 instead of 1 and 2.

Table 4.5A.18.4-2: RRCConnectionReconfiguration (step 10)

Derivation path: 36.508 table 4.6.1-8 using condition DRB-REL(8)

Table 4.5A.18.4-3: DEACTIVATE EPS BEARER CONTEXT REQUEST (step 10)

| Derivation Path: TS 36.508 Table 4.7.3-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS bearer identity | (identity of default EPS bearer associated with the PDN to be released) |  |  |
| ESM cause | 00011010 | Insufficient resources |  |

Table 4.5A.18.4-4: DEACTIVATE EPS BEARER CONTEXT ACCEPT (step 12)

| Derivation Path: TS 36.508 Table 4.7.3-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS bearer identity | (same as in DEACTIVATE EPS BEARER CONTEXT REQUEST of step 10) | The same value as the value set in DEACTIVATE EPS BEARER CONTEXT REQUEST message. |  |
| Procedure transaction identity | 0 | No procedure transaction identity assigned |  |

### 4.5A.19	Generic Test Procedure for IMS MO speech call establishment in E-UTRA / EVS

#### 4.5A.19.1	Initial conditions

See clause 4.5A.6.1.

#### 4.5A.19.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.19.3	Procedure

Table 4.5A.19.3-1: EUTRA/EPS signalling for IMS MO speech call / EVS

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U – S | Message |
| 1-7 | Same as table 4.5A.6.3-1, steps 1-7. |  |  |
| - | EXCEPTION: In parallel to the events described in steps 8 below, the behaviour in table 4.5A.19.3-2 occurs. (IMS MTSI MO speech call establishment) | - | - |
| 8 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer context. | --> | RRC: RRCConnectionReconfigurationComplete |
| 9-11 | Steps 3-4 expected sequence defined in annex C.44 of TS 34.229-1 [35]. MTSI MO speech call for EPS / EVS. | - | - |
| 12 | The SS configures a new RLC-UM data radio bearer with condition DRB (0,1), associated with the dedicated EPS bearer context. RRCConnectionReconfiguration message contains the ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message. EPS bearer context #4 (QCI 1) according to table 6.6.2-1: Reference dedicated EPS bearer contexts. | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST |
| - | EXCEPTION: In parallel to the events described in steps 13-14 below, the behaviour in table 4.5A.19.3-3 occurs. (IMS MTSI MO speech call establishment) | - | - |
| 13 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the dedicated EPS bearer. | --> | RRC: RRCConnectionReconfigurationComplete |
| 14 | The UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer NAS:ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |

Table 4.5A.19.3-2: Parallel behaviour

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Step 2 expected sequence defined in annex C.44 of TS 34.229-1 [35]. UE sends INVITE. | - | - |

Table 4.5A.19.3-3: Parallel behaviour

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1-9 | Steps 5-13 expected sequence defined in annex C.44 of TS 34.229-1 [35]. MTSI MO speech call for EPS / EVS. | - | - |

#### 4.5A.19.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7.

### 4.5A.20	Generic Test Procedure for IMS MT speech call establishment in E-UTRA / EVS

#### 4.5A.20.1	Initial conditions

See clause 4.5A.6.1.

#### 4.5A.20.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.20.3	Procedure

Table 4.5A.20.3-1: EUTRA/EPS signalling for IMS MT speech call / EVS

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U – S | Message |
| 1-8 | Same as table 4.5A.7.3-1, steps 1-8. |  |  |
| 9-11b | Steps 1-6 expected sequence defined in annex C.45 of TS 34.229-1 [35], MTSI MT speech call for EPS / EVS. | - | - |
| 12-14 | Same as table 4.5A.7.3-1, steps 13-15. |  |  |
| 15-16 | Void | - | - |
| 17-25 | Steps 7-15 expected sequence defined in annex C.45 of TS 34.229-1 [35], MTSI MT speech call for EPS / EVS. | - | - |
| 26 | Void | - | - |

#### 4.5A.20.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7.

### 4.5A.21	Generic Test Procedure for IMS MO Customized Alerting Tones and speech establishment in E-UTRA

#### 4.5A.21.1	Initial conditions

See clause 4.5A.6.1

#### 4.5A.21.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.21.3	Procedure

Table 4.5A.21.3-1: EUTRA/EPS signalling for IMS MO CAT and speech call

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1-12 | See table 4.5A.6.3-1, steps 1-12 | - | - |
| - | EXCEPTION: In parallel to the events described in steps 13-14 below, the behaviour in table 4.5A.21.3-2 occurs. (IMS MTSI MO CAT and speech call establishment) | - | - |
| 13-14 | See table 4.5A.6.3-1, steps 13-14 | - | - |

Table 4.5A.21.3-2: Parallel behaviour

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1-14 | Steps 5-18 expected sequence defined in 20.1.4 of TS 34.229-1 [35]. MTSI MO CAT and speech call for EPS. | - | - |

#### 4.5A.21.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7.

### 4.5A.22	Communication with the ProSe Function: Initial Access

#### 4.5A.22.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall be in Registered, Idle Mode state (State 2).

#### 4.5A.22.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.22.3	Procedure

The procedure is only applicable for pc_Provide_ProSe == true.

The HTTP signalling between the UE and the ProSe function is done over TLS i.e. the UE connects to TCP port 443 (HTTPS) and starts TLS handshake.

Table 4.5A.22.3-1: Communication with the ProSe Function

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The UE transmits an RRCConnectionRequest message with ' establishmentCause' set to 'mo-Data '. | --> | RRCConnectionRequest |
| 2 | SS transmit an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| 3 | The UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the session management procedure by including the SERVICE REQUEST message. | --> | RRC: RRCConnectionSetupComplete NAS: SERVICE REQUEST |
| 4 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |
| 5 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 6 | The SS configures a new data radio bearer, associated with the default EPS bearer context. The RRCConnectionReconfiguration message is using condition SRB2-DRB(1, 0). The DRB associated with default EPS bearer context obtained during the attach procedure is established | <-- | RRC: RRCConnectionReconfiguration |
| 7 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer context. | --> | RRC: RRCConnectionReconfigurationComplete |
| 8-10 | Steps 1-3 of procedure 4.5A.16.3. | - | - |
| - | EXCEPTION: In parallel to the event described in step 11 below, the behaviour described in Table 4.5A.22.3-2 takes place. | - | - |
| 11 | The UE transmits an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer NAS:ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |

Table 4.5A.22.3-2: Parallel behaviour

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that take place if the UE does not have a preconfigured IP address. | - | - |
| 1a1 | If initiated by the UE the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane. | - | - |
| 2 | The UE transmits a ClientHello message including PSK-based ciphersuites. | --> | TLS: ClientHello |
| 3 | The SS transmits the ServerHello, ServerKeyExchange and ServerHelloDone messages indicating GBA as required. | <-- | TLS: ServerHello TLS: ServerKeyExchange TLS: ServerHelloDone |
| 4-7 | The generic procedure for GBA authentication according TS 34.229-1 [43] subclause C.29.2 (steps 1-4) takes place. | - | - |
| 8 | The UE transmits the ClientKeyExchange including a PSK identity, ChangeCipherSpec and Finished messages. | --> | TLS: ClientKeyExchange TLS: ChangeCipherSpec TLS: Finished |
| 9 | The SS transmits the ChangeCipherSpec and Finished messages. | <-- | TLS: ChangeCipherSpec TLS: Finished |
| - | EXCEPTION: Steps 10a1 to 10d2 describe behaviour which depends on the ProSe Procedure being applied | - | - |
| 10a1 | IF ANNOUNCE/MONITOR REQUEST THEN The UE transmits a DISCOVERY_REQUEST message over the PC3 (UE to ProSe Function) interface. | --> | HTTP Request containing DISCOVERY_REQUEST |
| 10a2 | The SS transmits a DISCOVERY_RESPONSE message over the PC3 (UE to ProSe Function) interface. | <-- | HTTP Response containing DISCOVERY_RESPONSE |
| 10a3 | The UE transmits a SidelinkUEInformation message. | --> | SidelinkUEInformation |
| 10b1 | IF MATCH REPORT THEN The UE transmits a MATCH_REPORT message over the PC3 (UE to ProSe Function) interface. | --> | HTTP Request containing MATCH_REPORT |
| 10b2 | The SS transmits a MATCH_REPORT_ACK message over the PC3 (UE to ProSe Function) interface. | <-- | HTTP Response containing MATCH_REPORT_ACK |
| 10c1 | IF UEREGISTRATION THEN The UE transmits an UE_REGISTRATION_REQUEST message over the PC3 (UE to ProSe Function) interface. | --> | HTTP Request containing UE_REGISTRATION_REQUEST |
| 10c2 | The SS transmits an APPLICATION_REGISTRATION_RESPONSE message with a response-register. | <-- | HTTP Response containing UE_REGISTRATION_RESPONSE |
| 10d1 | IF USAGEINFOREPORT THEN The UE transmits an USAGE_INFORMATION_REPORT_LIST message over the PC3ch (UE to ProSe Function) interface. | --> | HTTP Request containing USAGE_INFORMATION_REPORT_LIST |
| 10d2 | SS sends USAGE_INFORMATION_REPORT_LIST_RESPONSE message. | <-- | HTTP Response containing USAGE_INFORMATION_REPORT_LIST_RESPONSE |

| Condition | Explanation |
| --- | --- |
| ANNOUNCE/MONITOR REQUEST | Steps applicable to Announce request procedure OR Monitor request procedure |
| MATCH REPORT | Steps applicable to Match report procedure |
| UEREGISTRATION | Steps applicable to EPC-level ProSe discovery / UE registration procedure |
| USAGEINFOREPORT | Steps applicable to usage information report list sending procedure. |

#### 4.5A.22.4	Specific message contents

HTTP specific message contents shall be referred to clause 4.7E.

ProSe specific message contents shall be referred to clause 4.7F.

TLS specific message contents shall be referred to clause 4.7H.

Further specific message contents shall be referred to clause 4.6 and 4.7.

### 4.5A.22A	Communication with the ProSe Function: Subsequent Access

#### 4.5A.22A.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall be in Registered Idle Mode state (State 2), or in state Generic RB Established (State 3).

-	The Communication with the ProSe Function: Initial Access in subclause 4.5A22 has taken place since the UE was switched on.

#### 4.5A.22A.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.22A.3	Procedure

The procedure is only applicable for pc_Provide_ProSe == true.

The HTTP signalling between the UE and the ProSe function is done over an already established TLS connection.

Table 4.5A.22A.3-1: Communication with the ProSe Function

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: Steps 1a1 to 1a7 describe behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that takes place if the UE is in RRC_IDLE state. | - | - |
| 1a1 | The UE transmits an RRCConnectionRequest message with ' establishmentCause' set to 'mo-Data '. | --> | RRCConnectionRequest |
| 1a 2 | SS transmit an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| 1a 3 | The UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the session management procedure by including the SERVICE REQUEST message. | --> | RRC: RRCConnectionSetupComplete NAS: SERVICE REQUEST |
| 1a 4 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |
| 1a 5 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 1a 6 | The SS configures two new data radio bearers, associated with the two default EPS bearer contexts. The RRCConnectionReconfiguration message is using condition SRB2-DRB(2, 0). | <-- | RRC: RRCConnectionReconfiguration |
| 1a 7 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearers, associated with the default EPS bearer contexts. | --> | RRC: RRCConnectionReconfigurationComplete |
| 2-10 | Void |  |  |
| - | EXCEPTION: Steps 10a1 to 10d2 describe behaviour which depends on the ProSe Procedure being applied | - | - |
| 10a1-10d2 | Steps 10a1-10d2 of Table 4.5A.22.3-2. | - | - |

#### 4.5A.22A.4	Specific message contents

HTTP specific message contents shall be referred to clause 4.7E.

ProSe specific message contents shall be referred to clause 4.7F.

Further specific message contents shall be referred to clause 4.6 and 4.7.

### 4.5A.23	Generic Test Procedure for IMS registration in EPC / WLAN

#### 4.5A.23.1	Initial conditions

System Simulator:

-	WLAN Cell 27 according to Table 4.4.8-1 with condition IMSoWLAN.

User Equipment:

-	The UE is in state Switched OFF (state 1).

#### 4.5A.23.2	Definition of system information messages

N/A

#### 4.5A.23.3	Procedure

Table 4.5A.23.3-1: IMS registration in EPC / WLAN

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The UE is switched on | - | - |
| 2 | The UE associates with the WLAN AP and obtains the local IP address | - | - |
| 3 | The UE performs a dynamic selection of ePDG using DNS query | - | - |
| 4 | The UE establishes an IPsec tunnel using the IKEv2 protocol as defined in 3GPP TS 33.402 [55] clause 8.2.2, Figure 8.2.2-1. | - | - |
| 5-13 | The UE performs the IMS registration procedure according TS 34.229-1 [43] subclause C.2c (steps 2-9). | - | - |

#### 4.5A.23.4	Specific message contents

None

### 4.5A.23A	Generic Test Procedure for IPsec Tunnel Disconnection in EPC / WLAN

#### 4.5A.23A.1	Initial conditions

System Simulator:

-	WLAN Cell 27 according to Table 4.4.8-1 with condition IMSoWLAN.

User Equipment:

-	The UE has established an ePDG Tunnel.

#### 4.5A.23A.2	Definition of system information messages

N/A

#### 4.5A.23A.3	Procedure

Table 4.5A.23A.3-1: Generic Test Procedure for IPsec Tunnel Disconnection in EPC / WLAN

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The UE initiates a disconnection from the existing IPsec tunnel as defined in 3GPP TS 24.302 [53] clause 7.2.4.1, which is acknowledged by the SS. |  |  |
| NOTE: It is assumed that the WLAN AP association remains throughout the procedure. |  |  |  |

#### 4.5A.23A.4	Specific message contents

None

### 4.5A.24	Generic Test Procedure for IMS emergency call establishment in EPC / WLAN

#### 4.5A.24.1	Initial conditions

System Simulator:

-	WLAN Cell 27 according to Table 4.4.8-1 with condition IMSoWLAN.

User Equipment:

-	The UE has selected an ePDG and is registered onto the IMS network according to clause 4.5A.23.

#### 4.5A.24.2	Definition of system information messages

N/A

#### 4.5A.24.3	Procedure

Table 4.5A.24.3-1: IMS emergency call establishment in EPC / WLAN

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The UE is triggered to initiate an emergency call | - | - |
| 2 | Void | - | - |
| 3 | The UE performs a dynamic selection of an ePDG that supports emergency services using DNS query | - | - |
| 4 | The UE establishes an additional IPsec tunnel using the IKEv2 protocol as defined in 3GPP TS 33.402 [55] clause 8.2.2, Figure 8.2.2-1. | - | - |
| NOTE 1: It is assumed that the WLAN AP association remains throughout the procedure. NOTE 2: This procedure is based on 3GPP Release 14. |  |  |  |

#### 4.5A.24.4	Specific message contents

None

### 4.5A.25	Generic Test Procedure for XCAP establishment in EPC / WLAN

This procedure is applicable only for the case when pc_WLAN_XCAP_without_PDN is set to false.

#### 4.5A.25.1	Initial conditions

System Simulator:

-	WLAN Cell 27 according to Table 4.4.8-1 with condition IMSoWLAN.

User Equipment:

-	The UE has selected an ePDG and is registered onto the IMS network according to clause 4.5A.23.

#### 4.5A.25.2	Definition of system information messages

N/A

#### 4.5A.25.3	Procedure

Table 4.5A.25.3-1: IMS XCAP establishment in EPC / WLAN

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Make the UE attempt XCAP signalling. | --> | - |
| 2 | The UE establishes an additional PDN and IPsec tunnel using the IKEv2 protocol as defined in 3GPP TS 33.402 [55] clause 8.2.2, Figure 8.2.2-1. | - | - |

#### 4.5A.25.4	Specific message contents

N/A

### 4.5A.26	Generic Test Procedure for eCall over IMS establishment in EUTRA: Normal Service

#### 4.5A.26.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE is in state Registered, Idle Mode (State 2)

#### 4.5A.26.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.26.3	Procedure

The establishment of eCall over IMS is assumed to always be mobile originated.

Note: The trigger to initiate MO call will be part of test case from where the generic procedure is called.

Table 4.5A.26.3-1: EUTRA/EPS signalling for eCall over IMS

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1-8a7 | Steps 3 to 10a7 of the generic radio bearer establishment procedure from TS 36.508 subclause 4.5.3.3-3 are executed | - | - |
| 9 | The UE transmits a PDN CONNECTIVITY REQUEST message to request an additional PDN, with 'Request type' set to 'emergency ('0100'B)'. | --> | NAS: PDN CONNECTIVITY REQUEST |
| 10 | The SS configures a new data radio bearer, associated with the default EPS bearer context. RRCConnectionReconfiguration message contains the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message. EPS bearer context #2 (QCI 5) according to table 6.6.1-1: Reference default EPS bearer context is used. Note: The APN is set to the test APN defined for emergency bearer services. | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST |
| - | EXCEPTION: In parallel to the events described in steps 11 to 15 below, the behaviour in table 4.5A.26.3-2 occurs. (Optional IP address allocation followed by IMS emergency registration and eCall over IMS establishment) | - | - |
| 11 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer for emergency call. | --> | RRC: RRCConnectionReconfigurationComplete |
| 12 | The UE transmits an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |
| 13 | The SS configures a new RLC-UM data radio bearer, associated with the dedicated EPS bearer context. RRCConnectionReconfiguration message contains the ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message. EPS bearer context #4 (QCI 1) according to table 6.6.2-1: Reference dedicated EPS bearer contexts is used. Note: the same PDN address is applicable because the linked EPS bearer ID refers to the default EBC allocated in step 2 | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST |
| 14 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer for emergency IMS signalling. | --> | RRC: RRCConnectionReconfigurationComplete |
| 15 | The UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |

Table 4.5A.26.3-2: Parallel behaviour

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: Step 1 describes behaviour that depends on the UE behaviour. | - | - |
| 1 | If initiated by the UE the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane. | - | - |
| 2-5 | Steps 1-4 expected sequence defined in annex C.20 of TS 34.229-1 [35]. Emergency registration procedure is performed. | - | - |
| 6-8 | Steps 1-3 defined in annex C.47 of TS 34.229-1 [35] Generic Test Procedure for NG eCall setup. | - | - |

#### 4.5A.26.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7 with the exceptions below.

Table 4.5A.26.4-1: PDN CONNECTIVITY REQUEST (step 9, Table 4.5A.26.3-1)

| Derivation Path: Table 4.7.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Request type |  | ‘0100’B | Emergency |  |
| Access point name |  | Not present or any value |  |  |

Table 4.5A.26.4-2: Message ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST (step 10, Table 4.5A.26.3-1)

| Derivation path: Table 4.7.3-6 and table 4.6.1-8 with condition AM-DRB-ADD(2) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Access point name | sos | APN value as recommended by IR.88 clause 6.4 [56] |  |
| EPS QoS | According to reference default EPS bearer context #2 – in table 6.6.1-1 | SS defines an additional dedicated EPS QoS |  |

Table 4.5A.26.4-3: Message ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST (step 13, Table 4.5A.26.3-1)

| Derivation path: Table 4.7.3-3 and table 4.6.1-8 with condition UM-DRB-ADD(3) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Linked EPS bearer identity | Default EBId-2 (same value like in table 4.5A.4.4-1) |  |  |
| EPS QoS | According to reference dedicated EPS bearer context #1 - in table 6.6.2-1 | SS defines an additional dedicated EPS QoS |  |
| TFT | According to reference dedicated EPS bearer context #1 - in table 6.6.2-1 |  |  |

### 4.5A.27	Generic Test Procedure for eCall over IMS establishment in EUTRA: eCall Only Support

#### 4.5A.27.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE is in state Switched OFF (State 1).

#### 4.5A.27.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.27.3	Procedure

The establishment of eCall over IMS is assumed to always be mobile originated.

Note: The generic procedure is defined as per TS 23.401 [24] clause 4.3.12.1.

Table 4.5A.27.3-1: EUTRA/EPS signalling for eCall over IMS (eCall only Support)

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The UE is switched on and SS waits 15 sec to allow the UE to camp on the serving cell and enter and remain in substate EMM-DEREGISTERED.eCALL-INACTIVE. | - | - |
| 2 | Make the UE attempt an eCall (See Note 1) | - | - |
| 3-18 | Steps 1 to 16 of the generic test procedure for UE registration sub clause 4.5.2.3. | - | - |
| 19 | The UE transmits a PDN CONNECTIVITY REQUEST message to request an additional PDN, with 'Request type' set to 'emergency ('0100'B)'. | --> | NAS: PDN CONNECTIVITY REQUEST |
| 20 | The SS configures a new data radio bearer, associated with the default EPS bearer context. RRCConnectionReconfiguration message contains the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message. EPS bearer context #2 (QCI 5) according to table 6.6.1-1: Reference default EPS bearer context is used. Note: The APN is set to the test APN defined for emergency bearer services. | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST |
| - | EXCEPTION: In parallel to the events described in steps 21 to 25 below, the behaviour in table 4.5A.27.3-2 occurs. (Optional IP address allocation followed by IMS emergency registration and eCall over IMS establishment) | - | - |
| 21 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer for emergency call. | --> | RRC: RRCConnectionReconfigurationComplete |
| 22 | The UE transmits an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |
| 23 | The SS configures a new RLC-UM data radio bearer, associated with the dedicated EPS bearer context. RRCConnectionReconfiguration message contains the ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message. EPS bearer context #4 (QCI 1) according to table 6.6.2-1: Reference dedicated EPS bearer contexts is used. Note: the same PDN address is applicable because the linked EPS bearer ID refers to the default EBC allocated in step 2 | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST |
| 24 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer for emergency IMS signalling. | --> | RRC: RRCConnectionReconfigurationComplete |
| 25 | The UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |
| Note 1: The request to originate a manual or Automatic eCall may be performed by MMI or AT command. |  |  |  |

Table 4.5A.27.3-2: Parallel behaviour

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: Step 1 describes behaviour that depends on the UE behaviour. | - | - |
| 1 | If initiated by the UE the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane. | - | - |
| 2-5 | Steps 1-4 expected sequence defined in annex C.20 of TS 34.229-1 [35]. Emergency registration procedure is performed. | - | - |
| 6-8 | Steps 1-3 defined in annex C47 of TS 34.229-1 [35] Generic procedure for NG eCall Setup. | - | - |

#### 4.5A.27.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7 with the exceptions below.

Table 4.5A.27.4-1: PDN CONNECTIVITY REQUEST (step 19)

| Derivation Path: Table 4.7.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Request type |  | ‘0100’B | Emergency |  |
| Access point name |  | Not present or any value |  |  |

Table 4.5A.27.4-2: Message ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST (step 20, Table 4.5A.27.3-1)

| Derivation path: Table 4.7.3-6 and table 4.6.1-8 with condition AM-DRB-ADD(2) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Access point name | sos | APN value as recommended by IR.88 clause 6.4 [56] |  |
| EPS QoS | According to reference default EPS bearer context #2 – in table 6.6.1-1 | SS defines an additional dedicated EPS QoS |  |

Table 4.5A.27.4-3: Message ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST (step 23, Table 4.5A.27.3-1)

| Derivation path: Table 4.7.3-3 and table 4.6.1-8 with condition UM-DRB-ADD(3) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Linked EPS bearer identity | Default EBId-2 (same value like in table 4.5A.4.4-1) |  |  |
| EPS QoS | According to reference dedicated EPS bearer context #1 - in table 6.6.2-1 | SS defines an additional dedicated EPS QoS |  |
| TFT | According to reference dedicated EPS bearer context #1 - in table 6.6.2-1 |  |  |

### 4.5A.28	Generic Test Procedure for Converged IP Communications establishment in EUTRA

#### 4.5A.28.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4.

User Equipment:

-	The UE is in state Switched OFF (state 1) according to [18].

#### 4.5A.28.2	Definition of system information messages

N/A

#### 4.5A.28.3	Procedure

Table 4.5A.28.3-1: Converged IP Communications service establishment in E-UTRA

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The UE is switched on |  |  |
| 2-16 | Steps 2-16 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - |
| 17-20 | Steps 1-4 of the generic procedure to establish an additional PDN Connection specified in TS 36.508 subclause 4.5A.16. |  |  |

#### 4.5A.28.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7 with the exceptions below.

Table 4.5A.26.4-1: PDN CONNECTIVITY REQUEST (step 17)

| Derivation Path: Table 4.7.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Access point name |  | Any allowed value | The UE includes a new APN. |  |

Table 4.5A.28.4-2: Message ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST (step 18)

| Derivation path: Table 4.6.1-8 with condition AM-DRB-ADD(1) for ADD_IMS or with condition AM-DRB-ADD(8) else |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS bearer identity | '0101'B | arbitrary value used for PDN connectivity being maintained during the test case | ADD_IMS_MultiMedia and SMS |
| EPS bearer identity | '1100'B | arbitrary value used for additional non-IMS PDN connectivity | ADD_IMS_Advanced_Messaging |
| Procedure transaction identity | PTI-1 | SS re-uses the particular PTI defined by UE for this present additional PDN connectivity request procedure |  |
| EPS QoS | See Reference default EPS bearer context #2 in table 6.6.1-1 |  |  |
| Access point name | APN as provided in ESM INFORMATION RESPONSE or PDN CONNECTIVITY REQUEST |  |  |

| Condition | Expression | Explanation |
| --- | --- | --- |
| ADD_IMS_MultiMedia and SMS | pc_IMS AND pc_Provide_IMS_as_second_APN | UE performs IMS registration for multi-media telephony and SMSoIP first, and IMS registration for advanced messaging services second. |
| ADD_IMS_Advanced_Messaging | pc_IMS AND NOT pc_Provide_IMS_as_second_APN | UE performs IMS registration for advanced messaging services first, and IMS registration for multi-media telephony and SMSoIP second. |

Table 4.5A.28.4-3: Message ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT (step 20)

| Derivation path: Table 4.7.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS bearer identity | (same value as used in step 2) |  |  |
| Procedure transaction identity | 0 | No procedure transaction identity assigned |  |

### 4.5A.29	Generic Test Procedure for Converged IP Communications establishment in EPC / WLAN

#### 4.5A.29.1	Initial conditions

System Simulator:

-	WLAN Cell 27 according to Table 4.4.8-1 in [18].

User Equipment:

-	The UE has selected an ePDG and is registered onto the IMS network according to 4.5A.23.

#### 4.5A.29.2	Definition of system information messages

N/A

#### 4.5A.29.3	Procedure

Table 4.5A.29.3-1: Converged IP Communications service establishment in EPC / WLAN

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The UE establishes an IPsec tunnel using the IKEv2 protocol as defined in 3GPP TS 33.402 [55] clause 8.2.2, Figure 8.2.2-1. | - | - |
| 2-9 | The UE performs the IMS registration procedure according TS 34.229-1 [43] subclause C.2c (steps 2-9). |  |  |
| NOTE: It is assumed that the WLAN AP association remains throughout the procedure. |  |  |  |

#### 4.5A.29.4	Specific message contents

FFS

### 4.5A.30	Generic Test Procedure for IMS Re-registration in E-UTRAN

The purpose of this procedure is to allow the successful completion of IMS Re-registration if it is initiated by the UE when moving from HPLMN to VPLMN and vice versa on E-UTRAN.

#### 4.5A.30.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

- 	The UE shall be in Registered, Idle Mode state (State 2).

#### 4.5A.30.2	Definition of system information messages

The default system information messages are used.

#### 4.5A.30.3	Procedure

Table 4.5A.30.3-1: Procedure for IMS Re-registration in E-UTRAN

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: Steps 1a1 to 1a2a10/1a2b1 describe behaviour that depends on the UE capability | - | - |
| 1a1 | IF pc_IMS then the SS starts timer Timer_1 = 10s. | - | - |
| - | EXCEPTION: Steps 1a2a1 to 1a2b1 describe a behaviour that depends on the UE implementation | - | - |
| 1a2a1-1a2a7 | Steps 3 to 9 of the generic radio bearer establishment procedure in TS 36.508 subclause 4.5.3.3 are executed. | - | - |
| 1a2a8-1a2a9 | The UE performs IMS re-registration using the generic procedure defined in 34.229-1 [35] Annex C.46. Note: SS cancels timer Timer_1 at step 1a2a8. | - | - |
| 1a2a10 | The SS releases the RRC connection. | - | - |
| 1a2b1 | The SS waits for Timer_1 expiry | - | - |

#### 4.5A.30.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7 with exceptions below.

REGISTER (Step 1a2a8)

Use the default message “REGISTER” in TS 34.229-1 [43] annex A.1.1 with conditions A2, A31 and (NOT A34).

## 4.6	Default RRC message and information elements contents

This clause contains the default values of common RRC messages and information elements, which unless indicated otherwise in specific clauses of TS 36.521-1 [21], TS 36.521-3 [34], TS 36.523-1 [18] and other clauses in this specification. All the messages and information elements are listed in alphabetical order.

### 4.6.1	Contents of RRC messages

#### –	CounterCheck

Table 4.6.1-0a: CounterCheck

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CounterCheck ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| counterCheck-r8 SEQUENCE { |  |  |  |
| drb-CountMSB-InfoList | Set according to specific message content |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	CounterCheckResponse

Table 4.6.1-0b: CounterCheckResponse

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CounterCheckResponse ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-UL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| counterCheckResponse-r8 SEQUENCE { |  |  |  |
| drb-CountInfoList | Set according to specific message content |  |  |
| nonCriticalExtension SEQUENCE {} | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	CSFBParametersRequestCDMA2000

Table 4.6.1-1: CSFBParametersRequestCDMA2000

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSFBParametersRequestCDMA2000 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| csfbParametersRequestCDMA2000-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE {} | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	CSFBParametersResponseCDMA2000

Table 4.6.1-2: CSFBParametersResponseCDMA2000

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSFBParametersResponseCDMA2000 ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| csfbParametersResponseCDMA2000-r8 SEQUENCE { |  |  |  |
| rand | Set according to specific message content |  |  |
| mobilityParameters | Set according to specific message content |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	DLInformationTransfer

Table 4.6.1-3: DLInformationTransfer

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DLInformationTransfer ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| dlInformationTransfer-r8 SEQUENCE { |  |  |  |  |
| dedicatedInfoType CHOICE { |  |  |  |  |
| dedicatedInfoNAS |  | Set according to specific message content |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	HandoverFromEUTRAPreparationRequest

Table 4.6.1-4: HandoverFromEUTRAPreparationRequest

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| HandoverFromEUTRAPreparationRequest ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| handoverFromEUTRAPreparationRequest-r8 SEQUENCE { |  |  |  |  |
| cdma2000-Type |  | type1XRTT |  |  |
| rand |  | A random value, generated by the SS |  |  |
| mobilityParameters |  | Set according to specific message content |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### -	LoggedMeasurementConfiguration

Table 4.6.1-4.0A: LoggedMeasurementConfiguration

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r10 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| loggedMeasurementConfiguration-r10 SEQUENCE { |  |  |  |  |
| traceReference-r10 SEQUENCE { |  |  |  |  |
| plmn-Identity-r10 SEQUENCE { |  |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit |  | See table 4.4.2-2 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit |  | See table 4.4.2-2 |  |  |
| } |  |  |  |  |
| traceId-r10 |  | ‘0EF’H | OCTET STRING (SIZE (3)) |  |
| } |  |  |  |  |
| traceRecordingSessionRef-r10 |  | ‘1A’H | OCTET STRING (SIZE (2)) |  |
| tce-Id-r10 |  | ‘5’H | OCTET STRING (SIZE (1)) |  |
| absoluteTimeInfo-r10 |  | Set to value corresponding to the absolute time when the message is sent | BIT STRING (SIZE (48)), see TS 36.331 clause 6.3.6 for format. |  |
| areaConfiguration-r10 |  | Not present |  |  |
| loggingDuration-r10 |  | min120 | ENUMERATED { min10, min20, min40, min60, min90, min120, spare2, spare1} |  |
| loggingInterval-r10 |  | ms10240 | ENUMERATED { ms1280, ms2560, ms5120, ms10240, ms20480, ms30720, ms40960, ms61440} |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### -	MasterInformationBlock-SL

Table 4.6.1-4A0: MasterInformationBlock-SL

| Derivation Path: 36.331 clause 6.5.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MasterInformationBlock-SL ::= SEQUENCE { |  |  |  |  |
| sl-Bandwidth-r12 |  | =the value of the corresponding field included in the preconfigured sidelink parameters (i.e. preconfigGeneral in SL-Preconfiguration), Table 6.8.1.1-1 |  |  |
| tdd-ConfigSL-r12 SEQUENCE { |  |  |  |  |
| subframeAssignmentSL-r12 |  | =the value of the corresponding field included in the preconfigured sidelink parameters (i.e. preconfigGeneral in SL-Preconfiguration, Table 6.8.1.1-1) |  |  |
| } |  |  |  |  |
| directFrameNumber-r12 |  | according to the subframe used to transmit the SLSS |  |  |
| directSubframeNumber-r12 |  | according to the subframe used to transmit the SLSS |  |  |
| inCoverage-r12 |  | TRUE | UE is in E-UTRAN coverage |  |
| reserved-r12 |  | 0000000000000000000 |  |  |
| } |  |  |  |  |

#### -	MasterInformationBlock-SL-V2X

Table 4.6.1-4A1: MasterInformationBlock-SL-V2X

| Derivation Path: 36.331 clause 6.5.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MasterInformationBlock-SL-V2X-r14 ::= SEQUENCE { |  |  |  |  |
| sl-Bandwidth-r14 |  | =the value of the corresponding field included in the preconfigured sidelink parameters (i.e. SL-V2X-PreconfigFreqInfo-r14 in SL-V2X-Preconfiguration, Table 4.10.1.1-1) |  |  |
| tdd-ConfigSL-r14 SEQUENCE { |  |  |  |  |
| subframeAssignmentSL-r12 |  | =the value of the corresponding field included in the preconfigured sidelink parameters (i.e. SL-V2X-PreconfigFreqInfo-r14 in SL-V2X-Preconfiguration, Table 4.10.1.1-1) |  |  |
| } |  |  |  |  |
| directFrameNumber-r14 |  | according to the subframe used to transmit the SLSS |  |  |
| directSubframeNumber-r14 |  | according to the subframe used to transmit the SLSS |  |  |
| inCoverage-r14 |  | TRUE | UE is in E-UTRAN coverage |  |
| reserved-r14 |  | 000000000000000000000000000 |  |  |
| } |  |  |  |  |

#### –	MBMSCountingRequest

Table 4.6.1-4AA: MBMSCountingRequest

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MBMSCountingRequest-r10 ::= SEQUENCE { |  |  |  |
| countingRequestList-r10 SEQUENCE (SIZE (1.. maxServiceCount)) OF SEQUENCE { |  |  |  |
| tmgi-r10 SEQUENCE { |  |  |  |
| plmn-Id-r9 CHOICE{ |  |  |  |
| plmn-Index-r9 | 1 |  |  |
| } |  |  |  |
| serviceId-r9 | ‘000000’O | MBMS service ID (TS 24.008 clause 10.5.6.13), OCTET STRING (SIZE (3)) |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |

#### –	MBMSCountingResponse

Table 4.6.1-4AB: MBMSCountingResponse

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MBMSCountingResponse-r10 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| countingResponse-r10 OF SEQUENCE { |  |  |  |
| mbsfn-AreaIndex-r10 | Not present |  |  |
| countingResponseList-r10 SEQUENCE (SIZE (1... maxServiceCount)) OF SEQUENCE { |  |  |  |
| countingResponseService-r10 | 0 |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	MBMSInterestIndication

Table 4.6.1-4AC: MBMSInterestIndication

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| interestIndication-r11 OF SEQUENCE { |  |  |  |
| mbms-FreqList-r11[n] SEQUENCE (SIZE (1..maxFreqMBMS-r11)) OF { INTEGER (0..maxEARFCN2) } | The number of entries is set according to specific message content. | INTEGER (0..maxEARFCN2) |  |
| mbms-Priority-r11 | Not checked | ENUMERATED {true} |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  | MBMS |
| nonCriticalExtension SEQUENCE { |  |  | SC-PTM |
| mbms-Services-r13 SEQUENCE (SIZE (0..maxMBMS-ServiceListPerUE-r13)) OF SEQUENCE { |  |  |  |
| tmgi-r13 SEQUENCE { |  |  |  |
| plmn-Id-r9 CHOICE { |  |  |  |
| plmn-Index-r9 | 1 |  |  |
| } |  |  |  |
| serviceId-r9 | ‘000001’H | OCTET STRING (SIZE (3)) |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| MBMS | MBMS cell environment |
| SC-PTM | SC-PTM cell environment |

#### –	MBSFNAreaConfiguration

Table 4.6.1-4A: MBSFNAreaConfiguration

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MBSFNAreaConfiguration-r9 ::= SEQUENCE { |  |  |  |
| commonSF-Alloc-r9 SEQUENCE (SIZE (1..maxMBSFN-Allocations)) OF SEQUENCE { |  |  |  |
| radioframeAllocationPeriod | n4 |  |  |
| radioframeAllocationOffset | 1 |  | FDD |
|  | 0 |  | TDD |
| subframeAllocation CHOICE { |  |  |  |
| oneFrame | ‘100000’B |  | FDD |
|  | ‘000010’B |  | TDD |
| } |  |  |  |
| } |  |  |  |
| commonSF-AllocPeriod-r9 | rf8 |  |  |
| pmch-InfoList-r9 SEQUENCE (SIZE (0..maxPMCH-PerMBSFN)) OF SEQUENCE { |  |  |  |
| pmch-Config-r9 SEQUENCE { |  |  |  |
| sf-AllocEnd-r9 | 1 |  |  |
| dataMCS-r9 | 0 |  |  |
| mch-SchedulingPeriod-r9 | rf8 |  |  |
| } |  |  |  |
| mbms-SessionInfoList-r9 SEQUENCE (SIZE (0..maxSessionPerPMCH)) OF SEQUENCE { |  |  |  |
| MBMS-SessionInfo-r9 SEQUENCE { |  |  |  |
| tmgi-r9 SEQUENCE { |  |  |  |
| plmn-Id-r9 CHOICE{ |  |  |  |
| plmn-Index-r9 | 1 |  |  |
| } |  |  |  |
| serviceId-r9 | ‘000000’O | OCTET STRING (SIZE (3)) |  |
| } |  |  |  |
| sessionId-r9 | Not present |  |  |
| logicalChannelIdentity-r9 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	MeasurementReport

Table 4.6.1-5: MeasurementReport

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |  |
| measResults |  | Set according to specific message content |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	MobilityFromEUTRACommand

Table 4.6.1-6: MobilityFromEUTRACommand

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MobilityFromEUTRACommand ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| mobilityFromEUTRACommand-r8 SEQUENCE { |  |  |  |  |
| csFallbackIndicator |  | Set according to specific message content |  |  |
| purpose CHOICE { |  | Set according to specific message content |  |  |
| handover |  |  |  |  |
| cellChangeOrder |  |  |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	Paging

Table 4.6.1-7: Paging

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |  |
| pagingRecordList SEQUENCE (SIZE (1..maxPageRec)) OF SEQUENCE { |  | 1 entry |  |  |
| ue-Identity[1] CHOICE { |  |  |  |  |
| s-TMSI |  | Set to the value of the S-TMSI of the UE |  |  |
| } |  |  |  |  |
| cn-Domain[1] |  | ps |  |  |
| } |  |  |  |  |
| systemInfoModification |  | Not present |  |  |
| etws-Indication |  | Not present |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |

#### –	RRCConnectionReconfiguration

Table 4.6.1-8: RRCConnectionReconfiguration

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  |  | Value/remark |  | Comment |  | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |  |  |  |  |
| rrc-TransactionIdentifier |  |  | RRC-TransactionIdentifier-DL |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |  |  |  |
| rrcConnectionReconfiguration-r8 ::= SEQUENCE { |  |  |  |  |  |  |  |
| measConfig |  |  | Not present |  |  |  |  |
|  |  |  | MeasConfig-DEFAULT |  |  |  | MEAS |
| mobilityControlInfo |  |  | Not present |  |  |  |  |
|  |  |  | MobilityControlInfo-HO |  |  |  | HO, HO-TO-EUTRA, HO-TO-EN-DC |
| dedicatedInfoNASList |  |  | Not present |  |  |  |  |
|  |  |  | Set according to specific message content |  |  |  | SRB2-DRB(n, m) DRB(n, m) SRB1-SRB2-DRB(n,m) AM-DRB-ADD(bid) UM-DRB-ADD(bid) DRB-REL(bid) DRB-Mod MCG_and_split EN-DC_SRB2-DRB |
| radioResourceConfigDedicated |  |  | Not present |  |  |  |  |
|  |  |  | RadioResourceConfigDedicated-SRB2-DRB(n, m) |  |  |  | SRB2-DRB(n, m) |
|  |  |  | RadioResourceConfigDedicated-DRB(n, m) |  |  |  | DRB(n, m) |
|  |  |  | RadioResourceConfigDedicated-AM-DRB-ADD(bid) |  |  |  | AM-DRB-ADD(bid) |
|  |  |  | RadioResourceConfigDedicated-UM-DRB-ADD(bid) |  |  |  | UM-DRB-ADD(bid) |
|  |  |  | RadioResourceConfigDedicated-DRB-REL(bid) |  |  |  | DRB-REL(bid) |
|  |  |  | RadioResourceConfigDedicated-HO |  |  |  | HO |
|  |  |  | RadioResourceConfigDedicated-HO-TO-EUTRA(n, m) |  |  |  | HO-TO-EUTRA(n,m)，,HO-TO-EN-DC(n,m) |
|  |  |  | RadioResourceConfigDedicated-DRB-Mod |  |  |  | DRB-Mod |
|  |  |  | RadioResourceConfigDedicated-SCell_AddMod |  |  |  | SCell_AddMod |
|  |  |  | RadioResourceConfigDedicated-PCell-PATTERN |  |  |  | PCell-PATTERN |
|  |  |  | RadioResourceConfigDedicated-EN-DC-Split |  |  |  | MCG_and_split |
|  |  |  | RadioResourceConfigDedicated-EN-DC_Split_DRB_Rel(bid) |  |  |  | EN-DC_Split_Rel(bid) |
|  |  |  | RadioResourceConfigDedicated-SRB4 |  |  |  | SRB4 |
|  |  |  | RadioResourceConfigDedicated-EN-DC-SRB2-DRB |  |  |  | EN-DC_SRB2-DRB |
| securityConfigHO |  |  | Not present |  |  |  |  |
|  |  |  | SecurityConfigHO-DEFAULT |  |  |  | HO, HO-TO-EUTRA，,HO-TO-EN-DC |
| nonCriticalExtension SEQUENCE { |  |  |  |  | RRCConnectionReconfiguration-v890-IEs |  |  |
| lateNonCriticalExtension SEQUENCE { |  |  | RRCConnectionReconfiguration-v8m0-IEs |  | 4Rx_PCC |  |
| lateNonCriticalExtension | Not present |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | RRCConnectionReconfiguration-v10i0-IEs |  |  |  |
| antennaInfoDedicatedPCell-v10i0 SEQUENCE { |  |  |  |  |  |  |
| maxLayersMIMO-r10 | fourLayers |  |  |  |  |  |
| } |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  | RRCConnectionReconfiguration-v10l0-IEs |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | RRCConnectionReconfiguration-v920-IEs |  |  |  |
| otherConfig-r9 | OtherConfig-r9 |  |  |  | OtherConfig |  |
| fullConfig-r9 | Not present |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | RRCConnectionReconfiguration-v1020-IEs |  |  |  |
| sCellToReleaseList-r10 | Not present |  |  |  |  |  |
| sCellToAddModList-r10 SEQUENCE (SIZE (1..maxSCell-r10)) OF | The same number of entries as the configured SCell(s) |  | n denotes the index of the entry |  | SCell_AddMod |  |
| sCellToAddMod-r10[n] | SCellToAddMod-r10-DEFAULT |  |  |  |  |  |
| } |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | RRCConnectionReconfiguration-v1130-IEs |  |  |  |
| systemInfomationBlockType1Dedicated-r11 | Not present |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | RRCConnectionReconfiguration-v1250-IEs |  |  |  |
| wlan-OffloadInfo-r12 CHOICE {} | Not present |  |  |  |  |  |
| wlan-OffloadInfo-r12 CHOICE { |  |  |  |  |  |  |
| release | NULL |  |  |  | WLAN-OffloadRelease |  |
| setup SEQUENCE { |  |  |  |  | WLAN-OffloadSetup |  |
| wlan-OffloadConfigDedicated-r12 | WLAN-OffloadConfig-DEFAULT |  |  |  |  |  |
| t350-r12 | min10 |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| scg-Configuration-r12 | Not present |  |  |  |  |  |
| scg-Configuration-r12 | SCG-Configuration-r12-DEFAULT |  |  |  | PSCell_Release, PSCell_Mod, PSCell_Add_Split_DRB, PSCell_Add_SCG_DRB |  |
| sl-SyncTxControl-r12 | Not present |  |  |  |  |  |
| sl-DiscConfig-r12 | Not present |  |  |  |  |  |
| sl-CommConfig-r12 | Not present |  |  |  |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  | RRCConnectionReconfiguration-v1310-IEs |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | RRCConnectionReconfiguration-v1310-IEs |  | LWA |  |
| lwa-configuration-r13 | Set as specified in test case |  |  |  |  |  |
| } |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | RRCConnectionReconfiguration-v1310-IEs |  | LWIP |  |
| lwip-configuration-r13 | Set as specified in test case |  |  |  |  |  |
| } |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | RRCConnectionReconfiguration-v1310-IEs |  | MCG_and_SCG, MCG_and _split, SCG-DRB(n,m), MCG_NR_PDCP, EN-DC_PSCell_Rel, SRB_NR_PDCP, EN-DC_EmbedNR_RRCRecon, EN-DC_Embed_RBConfig, EN-DC_PSCell_HO |  |
| nonCriticalExtension SEQUENCE { |  |  | RRCConnectionReconfiguration-v1430-IEs |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | RRCConnectionReconfiguration-v1510-IEs |  |  |  |
| nr-Config-r15 CHOICE { |  |  |  |  |  |  |
| Release |  |  |  |  | EN-DC_PSCell_Rel |  |
| setup SEQUENCE { |  |  |  |  | MCG_and_SCG, MCG_and_split, SCG-DRB(n,m), MCG_NR_PDCP, EN-DC_EmbedNR_RRCRecon, EN-DC_Embed_RBConfig, EN-DC_PSCell_HO |  |
| endc-ReleaseAndAdd-r15 | false |  |  |  |  |  |
| nr-SecondaryCellGroupConfig-r15 | OCTET STRING including the RRCReconfiguration message and the IE secondaryCellGroup according TS 38.508-1 [67], table 4.6.1-13 with condition EN-DC. |  |  |  | MCG_and_SCG，HO-TO-EN-DC(n,m) |  |
|  | OCTET STRING including the RRCReconfiguration message, the IE secondaryCellGroup according TS 38.508-1 [67], table 4.6.1-13 with condition EN-DC and the IE PDCP-Config according TS 38.508-1 [67], table 4.6.3-99 with condition Split. |  |  |  | MCG_and_split |  |
|  | OCTET STRING including the RRCReconfiguration message and the IE secondaryCellGroup according TS 38.508-1 [67], table [4.8.1-2]. |  |  |  | SCG-DRB(n,m) |  |
|  | OCTET STRING including the test case specific RRCReconfiguration |  |  |  | EN-DC_EmbedNR_RRCRecon |  |
|  | OCTET STRING including the RRCReconfiguration message and the IE secondaryCellGroup according TS 38.508-1 [67], table 4.6.1-13 with condition EN-DC_HO |  |  |  | EN-DC_PSCell_HO |  |
| p-MaxEUTRA-r15 | Not present |  |  |  |  |  |
|  | 23 |  |  |  | SIG AND EN-DC AND pc_dynamicPowerSharing |  |
|  | 20 |  |  |  | SIG AND EN-DC AND (NOT pc_dynamicPowerSharing) |  |
|  | 10 |  |  |  | RF AND EN-DC_Tx_PC3 |  |
|  | 13 |  |  |  | RF AND EN-DC_Tx_PC2 |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| sk-Counter-r15 | 0 |  |  |  | MCG_and_SCG, MCG_and _split, SCG-DRB(n,m), HO-TO-EN-DC（n,m） |  |
|  | Not Present |  |  |  |  |  |
| nr-RadioBearerConfig1-r15 | OCTET STRING including RadioBearerConfig according TS 38.508-1 [67], [table 4.6.3-132] with condition EN-DC_DRB. |  |  |  | MCG_and_SCG, MCG_and_split，HO-TO-EN-DC（n,m） |  |
|  | OCTET STRING including RadioBearerConfig according TS 38.508-1 [67], table [4.8.1-3]. |  |  |  | SCG-DRB(n,m) |  |
|  | OCTET STRING including RadioBearerConfig according TS 38.508-1 [67], table [4.6.3-132] with condition MCG_NR_PDCP |  |  |  | MCG_NR_PDCP |  |
|  | OCTET STRING including RadioBearerConfig according TS 38.508-1 [67], table 4.6.3-132 with condition SRB_NR_PDCP |  |  |  | SRB_NR_PDCP |  |
|  | OCTET STRING including the test case specific RadioBearerConfig |  |  |  | EN-DC_Embed_RBConfig |  |
|  | OCTET STRING including RadioBearerConfig according TS 38.508-1 [67], table 4.6.3-132 with conditions EN-DC_DRB and Re-establish_PDCP |  |  |  | EN-DC_PSCell_HO AND RBConfig_KeyChange |  |
|  | OCTET STRING including RadioBearerConfig according TS 38.508-1 [67], table 4.6.3-132 with conditions EN-DC_DRB and Recover_PDCP |  |  |  | EN-DC_PSCell_HO AND RBConfig_NoKeyChange |  |
| nr-RadioBearerConfig2-r15 | Not present |  |  |  |  |  |
| tdm-PatternConfig-r15 | Not present |  |  |  |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  | RRCConnectionReconfiguration-v1530-IEs |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| SRB2-DRB(n,m) | Establishment of a SRB and DRB combination with n x AM DRB and m x UM DRB (including establishment of SRB2) |
| DRB(n,m) | Establishment of additional n x AM DRB:s and m x UM DRB:s (SRB2 should already be established) |
| AM-DRB-ADD(bid) | Establishment of a single additional AM DRB with bearer identity bid (SRB2 should already be established) |
| UM-DRB-ADD(bid) | Establishment of a single additional UM DRB with bearer identity bid (SRB2 should already be established) |
| DRB-REL(bid) | Release of the DRB with bearer identity bid |
| HO | Intra LTE handover |
| MEAS | A measurement is configured |
| HO-TO-EUTRA(n,m) | Inter-RAT handover to E-UTRA including the establishment of a SRB1, SRB2 and n x AM DRB plus m x UM DRB |
| nonFullConfig | The field is not present in case of handover within E-UTRA when the fullConfig is included or in case of handover of handover to E-UTRA; otherwise it is optional present, need ON. |
| DRB-Mod | Modification of already established DRB ID 2 and is used for sending Modify EPS Bearer Context Request message (SRB2 should already be established) |
| SCell_AddMod | Addition or modification of SCell(s) |
| PCell-PATTERN | eICIC Serving Cell Pattern |
| OtherConfig | Other Configuration is configured |
| WLAN-OffloadSetup | When Dedicated WLAN Offload parameters need to be configured in UE |
| WLAN-OffloadRelease | When Dedicated WLAN Offload parameters need to be released in UE |
| PSCell_Mod | Modification of PSCell(s) |
| PSCell_Release | Release of PSCell |
| PSCell_Add_Split_DRB | Add PSCell(s) and setup of Split DRB |
| PSCell_Add_SCG_DRB | Add PSCell(s) and setup of SCG DRB |
| 4Rx_PCC | For 4Rx testcases with more than 2 Layers on PCC |
| MCG_and_SCG | EN-DC with MCG and SCG. |
| MCG_and_split | EN-DC with MCG and split. |
| SCG-DRB(n,m) | EN-DC establishment of additional n x AM DRB:s and m x UM DRB:s in SCG. |
| EN-DC_PSCell_Rel | Release of EN-DC PSCell |
| EN-DC_Split_Rel(bid) | Release of E-UTRA leg of EN-DC Split DRB with bearer identity bid |
| MCG_NR_PDCP | EN-DC configuration of MCG DRB with NR PDCP. |
| SRB_NR_PDCP | EN-DC configuration of SRB1 and SRB2 with NR PDCP. |
| EN-DC_SRB2-DRB | Establishment of SRB2 and DRB combination for EN-DC |
| EN-DC_EmbedNR_RRCRecon | To modify NR Cell Configuration in EN-DC by including RRCReconfiguration |
| EN-DC_EmbedNR_RBConfig | To modify NR RB Configuration in EN-DC by including RadioBearerConfig |
| SRB4 | Establishment of SRB4 |
| LWA | LWA is being configured |
| LWIP | LWIP is being configured |
| HO-TO-EN-DC(n,m) | Inter-RAT handover to EN-DC including the establishment of a SRB1, SRB2 and n x AM DRB plus m x UM DRB |
| EN-DC_PSCell_HO | RRCReconfiguration to perform PSCell handover (SCG change) in EN-DC |
| RBConfig_KeyChange | RadioBearerConfig to perform PSCell handover (SCG change) with security key change in EN-DC |
| RBConfig _NoKeyChange | RadioBearerConfig to perform PSCell handover (SCG change) without security key change in EN-DC |
| EN-DC_NR_CA | Addition, modification or release of NR SCell |
| EN-DC_Tx_PC3 | Power Class 3 UE testing Power Class 3 requirements for EN-DC transmitter test cases |
| EN-DC_Tx_PC2 | Power Class 2 UE testing Power Class 2 requirements for EN-DC transmitter test cases |

#### –	RRCConnectionReconfiguration (SideLink)

Table 4.6.1-8A: RRCConnectionReconfiguration (SideLink)

| Derivation Path: 36.508 table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sl-SyncTxControl-r12 SEQUENCE { |  |  |  |
| networkControlledSyncTx-r12 | off | This field indicates whether the UE shall transmit synchronisation information (i.e. become synchronisation source). Value On indicates the UE to transmit synchronisation information while value Off indicates the UE to not transmit such information. |  |
| } |  |  |  |
| sl-DiscConfig-r12 SEQUENCE { |  |  | DISC |
| discTxResources-r12 CHOICE { |  |  |  |
| release | NULL |  | RELEASE |
| setup CHOICE { |  |  | SETUP |
| scheduled-r12 SEQUENCE { |  |  | SCHEDULED |
| discTxConfig-r12 SEQUENCE { |  |  |  |
| cp-Len-r12 | normal |  |  |
| discPeriod-r12 | rf32 |  |  |
| numRetx-r12 | 0 |  |  |
| numRepetition-r12 | 1 |  |  |
| tf-ResourceConfig-r12 SEQUENCE { |  |  |  |
| prb-Num-r12 | 12 |  | BW5 |
|  | 24 |  | BW10 |
| prb-Start-r12 | 0 |  |  |
| prb-End-r12 | 23 |  | BW5 |
|  | 48 |  | BW10 |
| offsetIndicator-r12 | 160 | small-r12 | FDD |
|  | 163 |  | TDD |
| subframeBitmap-r12 | 00000000 11000000 00000000 00000000 00000000 | bs40-r12 | FDD |
|  | 00000000 11000000 | bs16-r12 | TDD |
| } |  |  |  |
| txParameters-r12 SEQUENCE { |  |  |  |
| txParametersGeneral-r12 SEQUENCE { |  |  |  |
| alpha-r12 | al0 |  |  |
| p0-r12 | 31 |  |  |
| } |  |  |  |
| ue-SelectedResourceConfig-r12 | Not present |  |  |
| } |  |  |  |
| rxParameters-r12 | Not present |  |  |
| } |  |  |  |
| discTF-IndexList-r12 SEQUENCE (SIZE (1..maxSL-TF-IndexPair-r12)) OF SL-TF-IndexPair-r12 { |  |  |  |
| SL-TF-IndexPair-r12[1] SEQUENCE { |  |  |  |
| discSF-Index-r12 | 1 | INTEGER (1.. 200) |  |
| discPRB-Index-r12 | 1 | INTEGER (1.. 50) |  |
| } |  |  |  |
| } |  |  |  |
| discHoppingConfig-r12 SEQUENCE { |  |  |  |
| a-r12 | 1 | INTEGER (1..200) Per cell parameter: see TS 36.213 [29, 14.3.1]. |  |
| b-r12 | 1 | INTEGER (1..10) Per UE parameter: see TS 36.213 [29, 14.3.1]. |  |
| c-r12 | n1 | ENUMERATED {n1, n5} Per cell parameter: see TS36.213 [29, 14.3.1] |  |
| } |  |  |  |
| } |  |  |  |
| ue-Selected-r12 SEQUENCE { |  |  | UE-SELECTED |
| discTxPoolDedicated-r12 SEQUENCE { |  |  |  |
| poolToReleaseList-r12 SEQUENCE (SIZE (1..maxSL-TxPool-r12)) OF SL-TxPoolIdentity-r12 { |  |  | RELEASE |
| SL-TxPoolIdentity-r12[1] | 1 |  |  |
| } |  |  |  |
| poolToAddModList-r12 SEQUENCE (SIZE (1..maxSL-TxPool-r12)) OF SL-DiscTxPoolToAddMod-r12 |  | 2 TxPools | SETUP |
| SL-DiscTxPoolToAddMod-r12[1] ::= SEQUENCE { |  | TxPool 1 |  |
| poolIdentity-r12 | 1 |  |  |
| pool-r12 SEQUENCE { |  |  |  |
| cp-Len-r12 | normal |  |  |
| discPeriod-r12 | rf32 |  |  |
| numRetx-r12 | 0 |  |  |
| numRepetition-r12 | 1 |  |  |
| tf-ResourceConfig-r12 SEQUENCE { |  |  |  |
| prb-Num-r12 | 12 |  | BW5 |
|  | 24 |  | BW10 |
| prb-Start-r12 | 0 |  |  |
| prb-End-r12 | 23 |  | BW5 |
|  | 48 |  | BW10 |
| offsetIndicator-r12 | 160 | small-r12 | FDD |
|  | 163 |  | TDD |
| subframeBitmap-r12 | 00000000 00110000 00000000 00000000 00000000 | bs40-r12 | FDD |
|  | 00000000 00110000 | bs16-r12 | TDD |
| } |  |  |  |
| txParameters-r12 SEQUENCE { |  |  |  |
| txParametersGeneral-r12 SEQUENCE { |  |  |  |
| alpha-r12 | al0 |  |  |
| p0-r12 | 31 |  |  |
| } |  |  |  |
| ue-SelectedResourceConfig-r12 SEQUENCE { |  |  |  |
| poolSelection-r12 SEQUENCE { |  | rsrpBased-r12 |  |
| threshLow-r12 | 3 | -90dBm |  |
| threshHigh-r12 | 4 | -80dBm |  |
| } |  |  |  |
| txProbability-r12 | p100 | 100% the probability of transmitting announcement in a discovery period when configured with a pool of resources |  |
| } |  |  |  |
| } |  |  |  |
| rxParameters-r12 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| SL-DiscTxPoolToAddMod-r12[2] ::= SEQUENCE { |  | TxPool 2 |  |
| poolIdentity-r12 | 2 |  |  |
| pool-r12 SEQUENCE { |  |  |  |
| cp-Len-r12 | normal |  |  |
| discPeriod-r12 | rf32 |  |  |
| numRetx-r12 | 0 |  |  |
| numRepetition-r12 | 1 |  |  |
| tf-ResourceConfig-r12 SEQUENCE { |  |  |  |
| prb-Num-r12 | 12 |  | BW5 |
|  | 24 |  | BW10 |
| prb-Start-r12 | 0 |  |  |
| prb-End-r12 | 23 |  | BW5 |
|  | 48 |  | BW10 |
| offsetIndicator-r12 | 160 | small-r12 | FDD |
|  | 163 |  | TDD |
| subframeBitmap-r12 | 00000000 00000011 00000000 00000000 00000000 | bs40-r12 | FDD |
|  | 00000000 00000011 | bs16-r12 | TDD |
| } |  |  |  |
| txParameters-r12 SEQUENCE { |  |  |  |
| txParametersGeneral-r12 SEQUENCE { |  |  |  |
| alpha-r12 | al0 |  |  |
| p0-r12 | 31 |  |  |
| } |  |  |  |
| ue-SelectedResourceConfig-r12 SEQUENCE { |  |  |  |
| poolSelection-r12 SEQUENCE { |  | rsrpBased-r12 |  |
| threshLow-r12 | 4 | -80dBm |  |
| threshHigh-r12 | 5 | -70dBm |  |
| } |  |  |  |
| txProbability-r12 | p100 | 100% the probability of transmitting announcement in a discovery period when configured with a pool of resources |  |
| } |  |  |  |
| } |  |  |  |
| rxParameters-r12 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| discTF-IndexList-v1260 CHOICE { |  |  |  |
| release | NULL |  |  |
| setup SEQUENCE { |  |  |  |
| discTF-IndexList-r12b SEQUENCE (SIZE (1..maxSL-TF-IndexPair-r12)) OF SL-TF-IndexPair-r12b |  |  |  |
| SL-TF-IndexPair-r12b[1] SEQUENCE { |  |  |  |
| discSF-Index-r12b | 1 | INTEGER (1.. 209) |  |
| discPRB-Index-r12b | 1 | INTEGER (1.. 49) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| sl-DiscConfig-r12 | Not Present |  | COMM |
| sl-CommConfig-r12 | Not Present |  | DISC |
| sl-CommConfig-r12 SEQUENCE { |  |  | COMM |
| commTxResources-r12 CHOICE { |  |  |  |
| release | NULL |  | RELEASE |
| setup CHOICE { |  |  | SETUP |
| scheduled-r12 SEQUENCE { |  |  | SCHEDULED |
| sl-RNTI-r12 | '1FF1'H |  |  |
| mac-MainConfig-r12 SEQUENCE { |  |  |  |
| periodic-BSR-TimerSL | sf20 |  |  |
| retx-BSR-TimerSL | sf320 |  |  |
| } |  |  |  |
| sc-CommTxConfig-r12 SEQUENCE { |  | SL-CommResourcePool-r12 Indicates a pool of resources for SC when E-UTRAN schedules Tx resources (i.e. when indices included in DCI format 5 indicate the actual data resources to be used as specified in TS 36.212 [61, 5.3.3.1.9]). |  |
| sc-CP-Len-r12 | normal |  |  |
| sc-Period-r12 | sf40 |  |  |
| sc-TF-ResourceConfig-r12 SEQUENCE { |  |  |  |
| prb-Num-r12 | 13 |  | BW5 |
|  | 25 |  | BW10 |
| prb-Start-r12 | 0 |  |  |
| prb-End-r12 | 24 |  | BW5 |
|  | 49 |  | BW10 |
| offsetIndicator-r12 | 0 | small-r12 |  |
| subframeBitmap-r12 | 00000000 11000000 00000000 00000000 00000000 | bs40-r12 | FDD |
| } |  |  |  |
| data-CP-Len-r12 | normal |  |  |
| dataHoppingConfig-r12 SEQUENCE { |  |  |  |
| hoppingParameter-r12 | 0 |  |  |
| numSubbands-r12 | ns1 |  |  |
| rb-Offset-r12 | 0 |  |  |
| } |  |  |  |
| ue-SelectedResourceConfig-r12 | Not present |  |  |
| rxParametersNCell-r12 | Not present |  |  |
| txParameters-r12 | Not present |  |  |
| } |  |  |  |
| mcs-r12 | 0 |  |  |
| } |  |  |  |
| ue-Selected-r12 SEQUENCE { |  |  | UE-SELECTED |
| commTxPoolNormalDedicated-r12 SEQUENCE { |  |  |  |
| poolToReleaseList-r12 SEQUENCE (SIZE (1..maxSL-TxPool-r12)) OF SL-TxPoolIdentity-r12 { |  |  | RELEASE |
| SL-TxPoolIdentity-r12[1] | 1 | INTEGER (1.. maxSL-TxPool-r12) |  |
| } |  |  |  |
| poolToAddModList-r12 SEQUENCE (SIZE (1..maxSL-TxPool-r12)) OF SL-CommTxPoolToAddMod-r12 |  | 1 TxPool | SETUP |
| SL-CommTxPoolToAddMod-r12[1] ::= SEQUENCE { |  | TxPool 1 |  |
| poolIdentity-r12 | 1 | INTEGER (1.. maxSL-TxPool-r12) |  |
| pool-r12 SEQUENCE { |  |  |  |
| sc-CP-Len-r12 | normal |  |  |
| sc-Period-r12 | sf40 |  |  |
| sc-TF-ResourceConfig-r12 SEQUENCE { |  |  |  |
| prb-Num-r12 | 13 |  | BW5 |
|  | 25 |  | BW10 |
| prb-Start-r12 | 0 |  |  |
| prb-End-r12 | 24 |  | BW5 |
|  | 49 |  | BW10 |
| offsetIndicator-r12 | 0 | small-r12 |  |
| subframeBitmap-r12 | 00000011 00000000 00000000 00000000 00000000 | bs40-r12 | FDD |
| } |  |  |  |
| data-CP-Len-r12 | normal |  |  |
| dataHoppingConfig-r12 SEQUENCE { |  |  |  |
| hoppingParameter-r12 | 0 |  |  |
| numSubbands-r12 | ns1 |  |  |
| rb-Offset-r12 | 0 |  |  |
| } |  |  |  |
| ue-SelectedResourceConfig-r12 SEQUENCE { |  |  |  |
| data-TF-ResourceConfig-r12 SEQUENCE { |  |  |  |
| prb-Num-r12 | 13 |  | BW5 |
|  | 25 |  | BW10 |
| prb-Start-r12 | 0 |  |  |
| prb-End-r12 | 24 |  | BW5 |
|  | 49 |  | BW10 |
| offsetIndicator-r12 | 0 | small-r12 |  |
| subframeBitmap-r12 | 00000000 00000000 00000011 11000000 00000000 | bs40-r12 | FDD |
| } |  |  |  |
| trpt-Subset-r12 | 001 |  |  |
| } |  |  |  |
| rxParametersNCell-r12 | Not present |  |  |
| txParameters-r12 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE {} |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| DISC | Only SideLink Discovery configuration provided |
| COMM | Only SideLink Communication configuration provided |
| RELEASE | Release Discovery/Communication Respurces |
| SETUP | Setup Discovery/Communication Respurces |
| SCHEDULED | The resources the UE can use are scheduled by the NW. |
| UE-SELECTED | The resources the UE can use are to be selected by the UE from set of resources provided by the NW. |
| FDD | FDD cell environment |
| TDD | TDD cell environment (TDD config 1) |

#### –	RRCConnectionReconfiguration (V2X)

Table 4.6.1-8B: RRCConnectionReconfiguration (V2X)

| Derivation Path: 36.508 table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sl-V2X-ConfigDedicated-r14 | SL-V2X-ConfigDedicated-r14-DEFAULT condition SCHEDULED and SETUP |  |  |
| sCellToAddModListExt-v14xy | Not present |  |  |
| perCC-GapIndicationRequest-r14 | Not present |  |  |
| systemInformationBlockType2Dedicated-r14 | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	RRCConnectionReconfigurationComplete

Table 4.6.1-9: RRCConnectionReconfigurationComplete

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionReconfigurationComplete ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-UL |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcConnectionReconfigurationComplete-r8 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | MCG_and_SCG, MCG_and_split, SCG-DRB(n,m),HO-TO-EN-DC(n,m) |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| scg-ConfigResponseNR-r15 |  | OCTET STRING including the RRCReconfigurationComplete message according TS 38.508-1 [67], table 4.6.1-14. |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| MCG_and_SCG | EN-DC with MCG and SCG. |
| MCG_and_split | EN-DC with MCG and split. |
| SCG-DRB(n,m) | EN-DC establishment of additional n x AM DRB:s and m x UM DRB:s in SCG. |
| HO-TO-EN-DC(n,m) | Inter-RAT handover to EN-DC including the establishment of a SRB1, SRB2 and n x AM DRB plus m x UM DRB |

#### –	RRCConnectionReestablishment

Table 4.6.1-10: RRCConnectionReestablishment

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionReestablishment ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| rrcConnectionReestablishment-r8 SEQUENCE { |  |  |  |  |
| radioResourceConfigDedicated |  | RadioResourceConfigDedicated-SRB1 |  |  |
| nextHopChainingCount |  | 0 |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	RRCConnectionReestablishmentComplete

Table 4.6.1-11: RRCConnectionReestablishmentComplete

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentComplete ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-UL |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcConnectionReestablishmentComplete-r8 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	RRCConnectionReestablishmentReject

Table 4.6.1-12: RRCConnectionReestablishmentReject

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentReject ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcConnectionReestablishmentReject-r8 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	RRCConnectionReestablishmentRequest

Table 4.6.1-13: RRCConnectionReestablishmentRequest

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentRequest ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcConnectionReestablishmentRequest-r8 SEQUENCE { |  |  |  |  |
| ue-Identity |  | Set according to specific message content |  |  |
| reestablishmentCause |  | Set according to specific message content |  |  |
| spare |  | Present but contents not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	RRCConnectionReject

Table 4.6.1-14: RRCConnectionReject

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionReject ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| rrcConnectionReject-r8 SEQUENCE { |  |  |  |  |
| waitTime |  | 3(seconds) |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	RRCConnectionRelease

Table 4.6.1-15: RRCConnectionRelease

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |  |
| releaseCause |  | other |  |  |
|  |  | rrc-Suspend |  | UP-CIoT |
| redirectedCarrierInfo |  | Not present |  |  |
| idleModeMobilityControlInfo |  | Not present |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | UP-CIoT |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| resumeIdentity-r13 |  | A valid value, calculated by the SS | BIT STRING (SIZE(40)) |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| UP-CIoT | User plane CIoT |

#### –	RRCConnectionRequest

Table 4.6.1-16: RRCConnectionRequest

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionRequest ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcConnectionRequest-r8 SEQUENCE { |  |  |  |  |
| ue-Identity CHOICE { |  |  |  |  |
| s-TMSI |  | Any allowed value |  |  |
| } |  |  |  |  |
| establishmentCause |  | Present but contents not checked |  |  |
| spare |  | Present but contents not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	RRCConnectionResume

Table 4.6.1-16A: RRCConnectionResume

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionResume-r13 ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| rrcConnectionResume-r13 SEQUENCE { |  |  |  |  |
| radioResourceConfigDedicated-r13 |  | RadioResourceConfigDedicated-SRB2-DRB(1, 0) |  |  |
| nextHopChainingCount-r13 |  | 0 |  |  |
| measConfig-r13 |  | Not present |  |  |
| antennaInfoDedicatedPCell-r13 |  | Not present |  |  |
| drb-ContinueROHC-r13 |  | Not present |  |  |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	RRCConnectionResumeComplete

Table 4.6.1-16B: RRCConnectionResumeComplete

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionResumeComplete-r13 ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-UL |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcConnectionResumeComplete-r13 SEQUENCE { |  |  |  |  |
| selectedPLMN-Identity-r13 |  | Not checked |  |  |
| dedicatedInfoNAS-r13 |  | Not checked |  |  |
| rlf-InfoAvailable-r13 |  | Not checked |  |  |
| logMeasAvailable-r13 |  | Not checked |  |  |
| connEstFailInfoAvailable-r13 |  | Not checked |  |  |
| mobilityState-r13 |  | Normal |  |  |
| mobilityHistoryAvail-r13 |  | Not checked |  |  |
| logMeasAvailableMBSFN-r13 |  | Not checked |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	RRCConnectionResumeRequest

Table 4.6.1-16C: RRCConnectionResumeRequest

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionResumeRequest-r13 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcConnectionResumeRequest-r13 SEQUENCE { |  |  |  |  |
| resumeIdentity-r13 CHOICE { |  |  |  |  |
| truncatedResumeID-r13 |  | Present but contents not checked |  |  |
| } |  |  |  |  |
| shortResumeMAC-I-r13 |  | Present but contents not checked |  |  |
| resumeCause-r13 |  | mt-Access |  |  |
| spare |  | Present but contents not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	RRCConnectionSetup

Table 4.6.1-17: RRCConnectionSetup

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionSetup ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| rrcConnectionSetup-r8 SEQUENCE { |  |  |  |  |
| radioResourceConfigDedicated |  | RadioResourceConfigDedicated-SRB1 |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	RRCConnectionSetupComplete

Table 4.6.1-18: RRCConnectionSetupComplete

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-UL |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |  |
| selectedPLMN-Identity |  | 1 |  |  |
| registeredMME |  | Not checked |  |  |
| dedicatedInfoNAS |  | Present but contents not checked |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | UP-CIoT or CP-CIoT |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| up-CIoT-EPS-Optimisation-r13 |  | true |  | UP-CIoT |
| cp-CIoT-EPS-Optimisation-r13 |  | true |  | CP-CIoT |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| UP-CIoT | If user plane CIoT supported (TS 36.523-2, A.4.4-1/132). Note 1: This condition is only applicable if also condition CIoT-test apply. |
| CP-CIoT | If control plane CIoT supported (TS 36.523-2, A.4.4-1/nnn). Note 1: This condition is only applicable if also condition CIoT-test apply. |

#### –	SCGFailureInformationNR

Table 4.6.1-18AA: SCGFailureInformationNR

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SCGFailureInformationNR-r15 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| sgsFailureInformationNR-r15-IEs SEQUENCE { |  |  |  |
| failureReportSCG-NR-r15 SEQUENCE { |  |  |  |
| failureType-r15 | Any allowed value | t310-Expiry, OR randomAccessProblem, OR rlc-MaxNumRetx, OR synchReconfigFailureSCG, OR scg-reconfigFailure, OR srb3-IntegrityFailure |  |
| measResultFreqListFailNR-r15 SEQUENCE (SIZE (1..maxFreqNR-r15)) OF SEQUENCE { | n entries of MeasResultFreqFailNR-r15 | n denotes the number of NR frequencies the UE is configured to measure by measConfig |  |
| MeasResultFreqFailNR-r15 SEQUENCE { | entry [1] |  |  |
| carrierFreq-r15 | Downlink ARFCN of ssbFrequency | ARFCN-ValueNR-r15 |  |
| measResultCellListNR-r15 SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | n entries of MeasResultCellNR-r15 | n denotes the number of reported cells/CSI-RS resources |  |
| MeasResultCellNR-r15 SEQUENCE { | entry [1] |  |  |
| PhysCellIdNR-r15 | INTEGER (0.. 1007) | the physical layer identity (PCI) of an NR cell |  |
| MeasResultNR-r15 SEQUENCE { |  |  |  |
| RSRP-RangeNR-r15 | as specified in TS 38.508-1 [67], Table 4.6.3-152 | Integer value for RSRP measurements |  |
| RSRQ-RangeNR-r15 | as specified in TS 38.508-1 [67], Table 4.6.3-153 | Integer value for RSRQ measurements |  |
| RS-SINR-RangeNR-r15 | as specified in TS 38.508-1 [67], Table 4.6.3-172 | Integer value for SINR measurements |  |
| } |  |  |  |
| MeasResultSSB-IndexList-r15 SEQUENCE (SIZE (1..maxRS-IndexReport-r15)) OF SEQUENCE { | n entries of MeasResultSSB-Index-r15 | n denotes the number of RS indices for RRM |  |
| MeasResultSSB-Index-r15 SEQUENCE { | entry [1] |  |  |
| RS-IndexNR-r15 | INTEGER (0.. maxRS-Index-1-r15) |  |  |
| MeasResultNR-r15 SEQUENCE { |  |  |  |
| RSRP-RangeNR-r15 | as specified in TS 38.508-1 [67], Table 4.6.3-152 | Integer value for RSRP measurements |  |
| RSRQ-RangeNR-r15 | as specified in TS 38.508-1 [67], Table 4.6.3-153 | Integer value for RSRQ measurements |  |
| RS-SINR-RangeNR-r15 | as specified in TS 38.508-1 [67], Table 4.6.3-172 | Integer value for SINR measurements |  |
| } |  |  |  |
| } |  |  |  |
| ... |  | MeasResultSSB-Index-r15 entry [x] if any |  |
| } |  |  |  |
| } |  |  |  |
| ... |  | MeasResultCellNR-r15 entry [x] if any |  |
| } |  |  |  |
| } |  |  |  |
| ... |  |  |  |
| } |  |  |  |
| measResultSCG-r15 | containing the NR MeasResultSCG-Failure IE as specified in TS 38.508-1 [67], Table 4.6.3-80 |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE {} |  |  |  |
| } |  |  |  |
| } |  |  |  |
| criticalExtensionsFuture SEQUENCE {} |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	SCPTMConfiguration

Table 4.6.1-18a: SCPTMConfiguration

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SCPTMConfiguration-r13 ::= SEQUENCE { |  |  |  |
| sc-mtch-InfoList-r13 SEQUENCE (SIZE (0..maxSC-MTCH-r13)) OF SEQUENCE { |  |  |  |
| mbmsSessionInfo-r13 SEQUENCE { |  |  |  |
| tmgi-r13 SEQUENCE { |  |  |  |
| plmn-Id-r9 CHOICE { |  |  |  |
| plmn-Index-r9 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| serviceId-r9 | ‘000001’H | OCTET STRING (SIZE (3)) |  |
| } |  |  |  |
| sessionId-r13 | Not present |  |  |
| } |  |  |  |
| g-RNTI-r13 | ‘0100’H |  |  |
| sc-mtch-schedulingInfo-r13 SEQUENCE { |  |  |  |
| on-DurationTimerSCPTM-r13 | Not present |  |  |
| drx-InactivityTimerSCPTM-r13 | Not present |  |  |
| schedulingPeriodStartOffsetSCPTM-r13 CHOICE{ |  |  |  |
| sf20 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| sc-mtch-neighbourCell-r13 | Not present |  |  |
| } |  |  |  |
| scptm-NeighbourCellList-r13 SEQUENCE (SIZE (1..maxNeighCell-SCPTM-r13)) OF SEQUENCE { |  |  |  |
| physCellId-r13 | Not present |  |  |
| carrierFreq-r13 | Not present |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |

#### –	SCPTMConfiguration-BR

Table 4.6.1-18b: SCPTMConfiguration-BR

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SCPTMConfiguration-BR-r14 ::= | SEQUENCE { |  |  |
| sc-mtch-InfoList-r14 SEQUENCE (SIZE (0..maxSC-MTCH-BR-r14)) OF SEQUENCE { |  |  |  |
| sc-mtch-CarrierFreq-r14 | FFS |  |  |
| mbmsSessionInfo-r14 SEQUENCE { |  |  |  |
| tmgi-r13 SEQUENCE { |  |  |  |
| plmn-Id-r9 CHOICE { |  |  |  |
| plmn-Index-r9 | 1 |  |  |
| } |  |  |  |
| serviceId-r9 | ‘000001’H | OCTET STRING (SIZE (3)) |  |
| } |  |  |  |
| sessionId-r13 | Not present |  |  |
| } |  |  |  |
| g-RNTI-r14 | ‘0100’H |  |  |
| sc-mtch-schedulingInfo-r14 | Not present |  |  |
| sc-mtch-neighbourCell-r14 | Not present |  |  |
| mpdcch-Narrowband-SC-MTCH-r14 | 1 |  |  |
| mpdcch-NumRepetition-SC-MTCH-r14 | r1 |  |  |
| mpdcch-StartSF-SC-MTCH-r14 CHOICE { |  |  |  |
| fdd-r14 | v1 |  | FDD |
| tdd-r14 | v1 |  | TDD |
| } |  |  |  |
| mpdcch-PDSCH-HoppingConfig-SC-MTCH-r14 | off |  |  |
| mpdcch-PDSCH-CEmodeConfig-SC-MTCH-r14 | ce-ModeA |  | CEmodeA |
|  | ce-ModeB |  | CEmodeB |
| mpdcch-PDSCH-MaxBandwidth-SC-MTCH-r14 | bw1dot4 |  |  |
| mpdcch-Offset-SC-MTCH-r14 | zero |  |  |
| p-a-r14 | Not present |  |  |
| } |  |  |  |
| scptm-NeighbourCellList-r13 SEQUENCE (SIZE (1..maxNeighCell-SCPTM-r13)) OF SEQUENCE { |  |  |  |
| physCellId-r13 | Not present |  |  |
| carrierFreq-r13 | Not present |  |  |
| } |  |  |  |
| p-b-r14 | Not present |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |
| CEmodeA | Used for CE mode A testing |
| CEmodeB | Used for CE mode B testing |

#### –	SecurityModeCommand

Table 4.6.1-19: SecurityModeCommand

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SecurityModeCommand ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| securityModeCommand-r8 SEQUENCE { |  |  |  |  |
| securityConfigSMC |  | SecurityConfigSMC-DEFAULT |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	SecurityModeComplete

Table 4.6.1-20: SecurityModeComplete

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SecurityModeComplete ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-UL |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| securityModeComplete-r8 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	SecurityModeFailure

Table 4.6.1-21: SecurityModeFailure

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SecurityModeFailure ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-UL |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| securityModeFailure-r8 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	SidelinkUEInformation

Table 4.6.1-21A: SidelinkUEInformation

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SidelinkUEInformation-r12 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| sidelinkUEInformation-r12 SEQUENCE { |  |  |  |
| commRxInterestedFreq-r12 | set according to specific message content. | INTEGER (0..maxEARFCN2) |  |
| commTxResourceReq-r12 SEQUENCE { |  |  |  |
| carrierFreq-r12 | set according to specific message content. | INTEGER (0..maxEARFCN2) |  |
| destinationInfoList-r12 SEQUENCE (SIZE (1.. maxSL-Dest-r12) OF SEQUENCE { | The number of entries is set according to specific message content. |  |  |
| SL-DestinationIdentity-r12 | set according to specific message content | BIT STRING (SIZE (24)) |  |
| } |  |  |  |
| } |  |  |  |
| discRxInterest-r12 | set according to specific message content. | ENUMERATED {true} |  |
| discTxResourceReq-r12 | set according to specific message content. | INTEGER (1..63) |  |
| lateNonCriticalExtension | Not Present |  |  |
| nonCriticalExtension SEQUENCE {} | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	SidelinkUEInformation (V2X)

Table 4.6.1-21B: SidelinkUEInformation (V2X)

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SidelinkUEInformation-r12 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| sidelinkUEInformation-r12 SEQUENCE { |  |  |  |
| commRxInterestedFreq-r12 | Not Present |  |  |
| commTxResourceReq-r12 | Not Present |  |  |
| discRxInterest-r12 | Not Present |  |  |
| discTxResourceReq-r12 | Not Present |  |  |
| lateNonCriticalExtension | Not Present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| v2x-CommRxInterestedFreqList-r14 | Not Present |  |  |
| v2x-CommRxInterestedFreqList-r14 SEQUENCE (SIZE (1..maxFreqV2X-r14)) OF INTEGER (0..maxFreqV2X-1-r14) | set according to specific message content | INTEGER (0..maxFreq) | COND_RX |
| p2x-CommTxType-r14 | Not Present |  |  |
| v2x-CommTxResourceReq-r14 | Not Present |  |  |
| v2x-CommTxResourceReq-r14 SEQUENCE (SIZE (1..maxFreqV2X-r14)) OF SEQUENCE { |  |  | COND_TX |
| carrierFreqCommTx-r14 | set according to specific message content |  |  |
| v2x-TypeTxSync-r14 | set according to specific message content |  |  |
| v2x-DestinationInfoList-r14 SEQUENCE (SIZE (1..maxSL-Dest-r12)) OF SEQUENCE { | The number of entries is set according to specific message content. |  |  |
| SL-DestinationIdentity-r12 | set according to specific message content | BIT STRING (SIZE (24)) |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| COND_RX | UE intends to receive V2X sidelink communication |
| COND_TX | UE intends to transmit V2X sidelink communication |

#### –	UECapabilityEnquiry

Table 4.6.1-22: UECapabilityEnquiry

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UECapabilityEnquiry ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| ueCapabilityEnquiry-r8 SEQUENCE { |  |  |  |  |
| ue-CapabilityRequest SEQUENCE (SIZE (1..maxRAT-Capabilities)) OF RAT-Type { |  | 1, 2 or 3 entries |  |  |
| RAT-Type[1] |  | eutra |  |  |
| RAT-Type[2] |  | eutra-nr |  | EN-DC |
| RAT-Type[3] |  | nr |  | NR, EN_DC |
| } |  |  |  |  |
| nonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | EN-DC, NR |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| requestedFreqBandsNR-MRDC-r15 |  | OCTET STRING including the FreqBandList IE according TS 38.508-1 [67], table 4.6.4-21. |  |  |
| nonCriticalExtension SEQUENCE |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| EN-DC | Used in E-UTRA-NR dual connectivity test cases defined in TS 38.523-1 [71]. |
| NR | Used in NR test cases defined in TS 38.523-1 [71]. |

#### –	UECapabilityInformation

Table 4.6.1-23: UECapabilityInformation

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  |  | Value/remark | Comment | Condition |
| UECapabilityInformation ::= SEQUENCE { |  |  |  |  |  |
| rrc-TransactionIdentifier |  |  | RRC-TransactionIdentifier-UL |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |  |
| ueCapabilityInformation-r8 SEQUENCE { |  |  |  |  |  |
| ue-CapabilityRAT-ContainerList SEQUENCE (SIZE (1..maxRAT-Capabilities)) OF SEQUENCE { |  |  | 1 entry |  |  |
| rat-Type |  |  | Set according to specific message content |  |  |
| ueCapabilityRAT-Container |  |  |  |  |  |
| ue-EUTRA-Capability SEQUENCE { |  |  |  |  |  |
| accessStratumRelease |  |  | Same as indicated in TC applicability in TS 36.523-2 [19] |  |  |
| ue-Category |  |  | Not checked |  |  |
| pdcp-Parameters SEQUENCE { |  |  |  |  |  |
| supportedROHC-Profiles SEQUENCE { |  |  |  |  |  |
| profile0x0001 |  |  | Not checked |  |  |
| profile0x0001 |  |  | true |  | ROHC |
| profile0x0002 |  |  | Not checked |  |  |
| profile0x0002 |  |  | true |  | ROHC |
| profile0x0003 |  |  | Not checked |  |  |
| profile0x0004 |  |  | Not checked |  |  |
| profile0x0006 |  |  | Not checked |  |  |
| profile0x0101 |  |  | Not checked |  |  |
| profile0x0102 |  |  | Not checked |  |  |
| profile0x0103 |  |  | Not checked |  |  |
| profile0x0104 |  |  | Not checked |  |  |
| } |  |  |  |  |  |
| maxNumberROHC-ContextSessions |  |  | Not checked |  |  |
| } |  |  |  |  |  |
| phyLayerParameters |  |  | Not checked |  |  |
| rf-Parameters |  |  | Not checked |  |  |
| measParameters |  |  | Not checked |  |  |
| featureGroupIndicators |  |  |  |  |  |
| -- FGI 1 |  |  | Not checked |  |  |
| -- FGI 2 |  |  | Not checked |  |  |
| -- FGI 3 |  |  | Not checked |  |  |
| -- FGI 4 |  |  | Not checked |  |  |
| -- FGI 5 |  |  | Not checked |  |  |
| -- FGI 6 |  |  | Not checked |  |  |
| -- FGI 7 |  |  | Not checked |  |  |
| -- FGI 8 |  |  | Not checked |  |  |
| -- FGI 9 |  |  | Not checked |  |  |
| -- FGI 10 |  |  | Not checked |  |  |
| -- FGI 11 |  |  | Not checked |  |  |
| -- FGI 12 |  |  | Not checked |  |  |
| -- FGI 13 |  |  | Not checked |  |  |
| -- FGI 14 |  |  | Not checked |  |  |
| -- FGI 15 |  |  | Not checked |  |  |
| -- FGI 16 |  |  | Not checked |  |  |
| -- FGI 17 |  |  | Not checked |  |  |
| -- FGI 18 |  |  | Not checked |  |  |
| -- FGI 19 |  |  | Not checked |  |  |
| -- FGI 20 |  |  | Not checked |  |  |
| -- FGI 21 |  |  | Not checked |  |  |
| -- FGI 22 |  |  | Not checked |  |  |
| -- FGI 23 |  |  | Not checked |  |  |
| -- FGI 24 |  |  | Not checked |  |  |
| -- FGI 25 |  |  | Not checked |  |  |
| -- FGI 26 |  |  | Not checked |  |  |
| -- FGI 27 |  |  | Not checked |  |  |
| -- FGI 28 |  |  | Not checked |  |  |
| -- FGI 29 |  |  | Not checked |  |  |
| -- FGI 30 |  |  | Not checked |  |  |
| -- FGI 31 |  |  | Not checked |  |  |
| -- FGI 32 |  |  | ‘0’B (Undefined) |  |  |
| interRAT-Parameters SEQUENCE { |  |  |  |  |  |
| utraFDD |  |  | Not checked |  |  |
| utraTDD128 |  |  | Not checked |  |  |
| utraTDD384 |  |  | Not checked |  |  |
| utraTDD768 |  |  | Not checked |  |  |
| geran |  |  | Not checked |  |  |
| cdma2000-HRPD |  |  | Not checked |  |  |
| cdma2000-1xRTT |  |  | Not checked |  |  |
| cdma2000-1xRTT |  |  | Not checked |  |  |
| } |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |
| phyLayerParameters-v920 |  |  | Not checked |  |  |
| interRAT-ParametersGERAN-v920 |  |  | Not checked |  |  |
| interRAT-ParametersUTRA-v920 |  |  | Not checked |  |  |
| interRAT-ParametersCDMA2000-v920 |  |  | Not checked |  |  |
| deviceType-r9 |  |  | Not checked |  |  |
| csg-ProximityIndicationParameters-r9 |  |  | Not checked |  |  |
| neighCellSI-AcquisitionParameters-r9 |  |  | Not checked |  |  |
| son-Parameters-r9 |  |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |
| lateNonCriticalExtension SEQUENCE { |  |  |  |  |  |
| featureGroupIndRel9Add-r9 |  |  |  |  |  |
| -- FGI 33 |  |  | Not checked |  |  |
| -- FGI 34 |  |  | Not checked |  |  |
| -- FGI 35 |  |  | Not checked |  |  |
| -- FGI 36 |  |  | Not checked |  |  |
| -- FGI 37 |  |  | Not checked |  |  |
| -- FGI 38 |  |  | Not checked |  |  |
| -- FGI 39 |  |  | Not checked |  |  |
| -- FGI 40 |  |  | Not checked |  |  |
| -- FGI 41 |  |  | Not checked |  |  |
| -- FGI 42-64 |  |  | ‘0’B (Undefined) |  |  |
| fdd-Add-UE-EUTRA-Capabilities-r9 SEQUENCE { |  |  |  |  |  |
| phyLayerParameters-r9 |  |  | Not checked |  |  |
| featureGroupIndicators-r9 |  |  |  | BITSTRING 32 |  |
| -- FGI 1F |  |  | Not checked |  |  |
| -- FGI 2F |  |  | Not checked |  |  |
| -- FGI 3F |  |  | Not checked |  | FDD = TDD |
| -- FGI 4F |  |  | Not checked |  |  |
| -- FGI 5F |  |  | Not checked |  | FDD = TDD |
| -- FGI 6F |  |  | Not checked |  | FDD = TDD |
| -- FGI 7F |  |  | Not checked |  | FDD = TDD |
| -- FGI 8F |  |  | Not checked |  |  |
| -- FGI 9F |  |  | Not checked |  |  |
| -- FGI 10F |  |  | Not checked |  |  |
| -- FGI 11F |  |  | Not checked |  |  |
| -- FGI 12F |  |  | Not checked |  |  |
| -- FGI 13F |  |  | Not checked |  | FDD = TDD |
| -- FGI 14F |  |  | Not checked |  | FDD = TDD |
| -- FGI 15F |  |  | Not checked |  |  |
| -- FGI 16F |  |  | Not checked |  | FDD = TDD |
| -- FGI 17F |  |  | Not checked |  | FDD = TDD |
| -- FGI 18F |  |  | Not checked |  | FDD = TDD |
| -- FGI 19F |  |  | Not checked |  |  |
| -- FGI 20F |  |  | Not checked |  | FDD = TDD |
| -- FGI 21F |  |  | Not checked |  | FDD = TDD |
| -- FGI 22F |  |  | Not checked |  |  |
| -- FGI 23F |  |  | Not checked |  |  |
| -- FGI 24F |  |  | Not checked |  |  |
| -- FGI 25F |  |  | Not checked |  | FDD = TDD |
| -- FGI 26F |  |  | Not checked |  |  |
| -- FGI 27F |  |  | Not checked |  |  |
| -- FGI 28F |  |  | Not checked |  |  |
| -- FGI 29F |  |  | Not checked |  |  |
| -- FGI 30F |  |  | Not checked |  | FDD = TDD |
| -- FGI 31F |  |  | Not checked |  | FDD = TDD |
| -- FGI 32F |  |  | ‘0’B (Undefined) if present |  |  |
| featureGroupIndRel9Add-r9 |  |  |  | BITSTRING 32 |  |
| -- FGI 33F |  |  | Not checked |  |  |
| -- FGI 34F |  |  | Not checked |  |  |
| -- FGI 35F |  |  | Not checked |  |  |
| -- FGI 36F |  |  | Not checked |  |  |
| -- FGI 37F |  |  | Not checked |  |  |
| -- FGI 38F |  |  | Not checked |  |  |
| -- FGI 39F |  |  | Not checked |  |  |
| -- FGI 40F |  |  | Not checked |  |  |
| -- FGI 41F |  |  | Not checked |  |  |
| -- FGI 42F-64F |  |  | ‘0’B (Undefined) if present |  |  |
| interRAT-ParametersGERAN-r9 |  |  | Not checked |  |  |
| interRAT-ParametersUTRA-r9 |  |  | Not checked |  |  |
| interRAT-ParametersGERAN-r9 |  |  | Not checked |  |  |
| interRAT-ParametersCDMA2000-r9 |  |  | Not checked |  |  |
| neighCellSI-AcquisitionParameters-r9 |  |  | Not checked |  |  |
| } |  |  |  |  |  |
| tdd-Add-UE-EUTRA-Capabilities-r9 SEQUENCE { |  |  |  |  |  |
| phyLayerParameters-r9 |  |  | Not checked |  |  |
| featureGroupIndicators-r9 |  |  |  | BITSTRING 32 |  |
| -- FGI 1T |  |  | Not checked |  |  |
| -- FGI 2T |  |  | Not checked |  |  |
| -- FGI 3T |  |  | Not checked |  | FDD = TDD |
| -- FGI 4T |  |  | Not checked |  |  |
| -- FGI 5T |  |  | Not checked |  | FDD = TDD |
| -- FGI 6T |  |  | Not checked |  | FDD = TDD |
| -- FGI 7T |  |  | Not checked |  | FDD = TDD |
| -- FGI 8T |  |  | Not checked |  |  |
| -- FGI 9T |  |  | Not checked |  |  |
| -- FGI 10T |  |  | Not checked |  |  |
| -- FGI 11T |  |  | Not checked |  |  |
| -- FGI 12T |  |  | Not checked |  |  |
| -- FGI 13T |  |  | Not checked |  | FDD = TDD |
| -- FGI 14T |  |  | Not checked |  | FDD = TDD |
| -- FGI 15T |  |  | Not checked |  |  |
| -- FGI 16T |  |  | Not checked |  | FDD = TDD |
| -- FGI 17T |  |  | Not checked |  | FDD = TDD |
| -- FGI 18T |  |  | Not checked |  | FDD = TDD |
| -- FGI 19T |  |  | Not checked |  |  |
| -- FGI 20T |  |  | Not checked |  | FDD = TDD |
| -- FGI 21T |  |  | Not checked |  | FDD = TDD |
| -- FGI 22T |  |  | Not checked |  |  |
| -- FGI 23T |  |  | Not checked |  |  |
| -- FGI 24T |  |  | Not checked |  |  |
| -- FGI 25T |  |  | Not checked |  | FDD = TDD |
| -- FGI 26T |  |  | Not checked |  |  |
| -- FGI 27T |  |  | Not checked |  |  |
| -- FGI 28T |  |  | Not checked |  |  |
| -- FGI 29T |  |  | Not checked |  |  |
| -- FGI 30T |  |  | Not checked |  | FDD = TDD |
| -- FGI 31T |  |  | Not checked |  | FDD = TDD |
| -- FGI 32T |  |  | ‘0’B (Undefined) if present |  |  |
| featureGroupIndRel9Add-r9 |  |  |  | BITSTRING 32 |  |
| -- FGI 33T |  |  | Not checked |  |  |
| -- FGI 34T |  |  | Not checked |  |  |
| -- FGI 35T |  |  | Not checked |  |  |
| -- FGI 36T |  |  | Not checked |  |  |
| -- FGI 37T |  |  | Not checked |  |  |
| -- FGI 38T |  |  | Not checked |  |  |
| -- FGI 39T |  |  | Not checked |  |  |
| -- FGI 40T |  |  | Not checked |  |  |
| -- FGI 41T |  |  | Not checked |  |  |
| -- FGI 42T-64T |  |  | ‘0’B (Undefined) if present |  |  |
| interRAT-ParametersGERAN-r9 |  |  | Not checked |  |  |
| interRAT-ParametersUTRA-r9 |  |  | Not checked |  |  |
| interRAT-ParametersGERAN-r9 |  |  | Not checked |  |  |
| interRAT-ParametersCDMA2000-r9 |  |  | Not checked |  |  |
| neighCellSI-AcquisitionParameters-r9 |  |  | Not checked |  |  |
| } |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | Not checked |  |  |
| interRAT-ParametersUTRA-v9c0 |  |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |
| phyLayerParameters-v9d0 |  |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |
| rf-Parameters-v9e0 |  |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE {} |  |  | Not checked |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |
| ue-Category-v1020 |  |  | Not checked |  |  |
| phyLayerParameters-v1020 |  |  | Not checked |  |  |
| rf-Parameters-v1020 |  |  | Not checked |  |  |
| measParameters-v1020 |  |  | Not checked |  |  |
| featureGroupIndRel10-r10 |  |  |  | BITSTRING 32 |  |
| -- FGI 101 |  |  | Not checked |  |  |
| -- FGI 102 |  |  | Not checked |  |  |
| -- FGI 103 |  |  | Not checked |  |  |
| -- FGI 104 |  |  | Not checked |  |  |
| -- FGI 105 |  |  | Not checked |  |  |
| -- FGI 106 |  |  | Not checked |  |  |
| -- FGI 107 |  |  | Not checked |  |  |
| -- FGI 108 |  |  | Not checked |  |  |
| -- FGI 109 |  |  | Not checked |  |  |
| -- FGI 110 |  |  | Not checked |  |  |
| -- FGI 111 |  |  | Not checked |  |  |
| -- FGI 112 |  |  | Not checked |  |  |
| -- FGI 113 |  |  | Not checked |  |  |
| -- FGI 114 |  |  | Not checked |  |  |
| -- FGI 115 |  |  | Not checked |  |  |
| -- FGI 116 |  |  | Not checked |  |  |
| -- FGI 117-132 |  |  | ‘0’B (Undefined) if present |  |  |
| interRAT-ParametersCDMA2000-v1020 |  |  | Not checked |  |  |
| ue-BasedNetwPerfMeasParameters-r10 |  |  | Not checked |  |  |
| interRAT-ParametersUTRA-TDD-v1020 |  |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  | Not checked |  |  |
| fdd-Add-UE-EUTRA-Capabilities-v1060 |  |  | Not checked |  |  |
| tdd-Add-UE-EUTRA-Capabilities-v1060 |  |  | Not checked |  |  |
| rf-Parameters-v1060 |  |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |
| rf-Parameters-v1090 |  |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |
| pdcp-Parameters-v1130 |  |  | Not checked |  |  |
| phyLayerParameters-v1130 |  |  | Not checked |  |  |
| rf-Parameters-v1130 |  |  | Not checked |  |  |
| measParameters-v1130 |  |  | Not checked |  |  |
| interRAT-ParametersCDMA2000-v1130 |  |  | Not checked |  |  |
| otherParameters-r11 |  |  | Not checked |  |  |
| fdd-Add-UE-EUTRA-Capabilities-v1130 |  |  | Not checked |  |  |
| tdd-Add-UE-EUTRA-Capabilities-v1130 |  |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  | Not checked |  |  |
| phyLayerParameters-v1170 |  |  | Not checked |  |  |
| ue-Category-v1170 |  |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |
| rf-Parameters-v1180 |  |  | Not checked |  |  |
| mbms-Parameters-r11 |  |  | Not checked |  |  |
| fdd-Add-UE-EUTRA-Capabilities-v1180 |  |  | Not checked |  |  |
| tdd-Add-UE-EUTRA-Capabilities-v1180 |  |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |
| ue-Category-v11a0 |  |  | Not checked |  |  |
| measParameters-v11a0 |  |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |
| phyLayerParameters-v1250 |  |  | Not checked |  |  |
| rf-Parameters-v1250 |  |  | Not checked |  |  |
| rlc-Parameters-r12 |  |  | Not checked |  |  |
| ue-BasedNetwPerfMeasParameters-v1250 |  |  | Not checked |  |  |
| ue-Category-v1250 |  |  | Not checked |  |  |
| wlan-IW-Parameters-r12 |  |  | Not checked |  |  |
| measParameters-v1250 |  |  | Not checked |  |  |
| dc-Parameters-r12 |  |  | Not checked |  |  |
| mbms-Parameters-v1250 |  |  | Not checked |  |  |
| mac-Parameters-r12 |  |  | Not checked |  |  |
| fdd-Add-UE-EUTRA-Capabilities-v1250 |  |  | Not checked |  |  |
| tdd-Add-UE-EUTRA-Capabilities-v1250 |  |  | Not checked |  |  |
| sl-Parameters-r12 |  |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE {} |  |  |  |  |  |
| ue-CategoryDL-v1260 |  | Not checked |  |  |  |
| nonCriticalExtension { |  |  |  |  |  |
| rf-Parameters-v1270 |  | Not checked |  |  |  |
| nonCriticalExtension { |  |  |  |  |  |
| phyLayerParameters-v1280 |  | Not checked |  |  |  |
| nonCriticalExtension { |  |  |  |  |  |
| phyLayerParameters-v1280 |  | Not checked |  |  |  |
| nonCriticalExtension { |  |  |  |  |  |
| ue-CategoryDL-v1310 |  | Not checked |  |  |  |
| ue-CategoryUL-v1310 |  | Not checked |  |  |  |
| pdcp-Parameters-v1310 |  | Not checked |  |  |  |
| rlc-Parameters-v1310 |  | Not checked |  |  |  |
| mac-Parameters-v1310 |  | Not checked |  |  |  |
| phyLayerParameters-v1310 |  | Not checked |  |  |  |
| rf-Parameters-v1310 |  | Not checked |  |  |  |
| measParameters-v1310 |  | Not checked |  |  |  |
| dc-Parameters-v1310 |  | Not checked |  |  |  |
| sl-Parameters-v1310 |  | Not checked |  |  |  |
| scptm-Parameters-r13 |  | Not checked |  |  |  |
| ce-Parameters-r13 |  | Not checked |  |  |  |
| interRAT-ParametersWLAN-r13 |  | Not checked |  |  |  |
| laa-Parameters-r13 |  | Not checked |  |  |  |
| lwa-Parameters-r13 |  | Not checked |  |  |  |
| wlan-IW-Parameters-v1310 |  | Not checked |  |  |  |
| lwip-Parameters-r13 |  | Not checked |  |  |  |
| fdd-Add-UE-EUTRA-Capabilities-v1310 |  | Not checked |  |  |  |
| tdd-Add-UE-EUTRA-Capabilities-v1310 |  | Not checked |  |  |  |
| nonCriticalExtension { |  |  |  |  |  |
| ce-Parameters-v1320 |  | Not checked |  |  |  |
| rf-Parameters-v1320 |  | Not checked |  |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not checked |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  | Aerial UE |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |
| UE-EUTRA-Capability-v1530-IEs SEQUENCE { |  |  |  |  |  |
| measParameters-v1530 SEQUENCE { |  |  |  |  |  |
| heightMeas-r15 |  | Not checked |  |  |  |
| multipleCellsMeasExtension-r15 |  | Not checked |  |  |  |
| } |  |  |  |  |  |
| phyLayerParameters-v1530 SEQUENCE { |  |  |  |  |  |
| ul-PowerControlEnhancements-r15 |  | Not checked |  |  |  |
| } |  |  |  |  |  |
| otherParameters-v1530 SEQUENCE { |  |  |  |  |  |
| flightPathPlan-r15 |  | Not checked |  |  |  |
| } |  |  |  |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not checked |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| nonCriticalExtension SEQUENCE {} |  |  | Not checked |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| ROHC | Support of ROHC profile0x0001 and ROHC profile0x0002 (TS 36.523-2 table A.4.4-1/40 and A.4.4-1/41). |
| FDD = TDD | UE is not allowed to signal different values for FDD and TDD, is not checked per default |
| Aerial UE | For Aerial vehicles |

#### –	UEInformationRequest

Table 4.6.1-23A: UEInformationRequest

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationRequest-r9 ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| ueInformationRequest-r9 SEQUENCE { |  |  |  |  |
| rach-ReportReq-r9 |  | FALSE |  |  |
| rlf-ReportReq-r9 |  | FALSE |  |  |
|  |  | TRUE |  | RLF report |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | Logged MDT, ConEstFail |
| logMeasReportReq-r10 |  | true |  | Logged MDT |
|  |  | Not present |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| connEstFailReportReq-r11 |  | true |  | ConEstFail |
|  |  | Not present |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| Logged MDT | Used for Logged MDT test cases |
| ConEstFail | Used for RRC connection establishment failure report related test cases |
| RLF report | Used for Radio Link Failure Logging test cases |

#### –	UEInformationResponse

Table 4.6.1-23B: UEInformationResponse

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-UL |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |  |
| rach-Report-r9 SEQUENCE {} |  | Not checked |  |  |
| rlf-Report-r9 SEQUENCE {} |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| logMeasReport-r10 SEQUENCE {} |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### -	ULHandoverPreparationTransfer

Table 4.6.1-24: ULHandoverPreparationTransfer

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ULHandoverPreparationTransfer ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ulHandoverPreparationTransfer-r8 SEQUENCE { |  |  |  |
| cdma2000-Type | type1XRTT |  |  |
| meid | Set to the 56 bit CDMA2000 mobile identification number of the UE |  |  |
| dedicatedInfo | Set according to specific message content |  |  |
| nonCriticalExtension SEQUENCE {} | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	ULInformationTransfer

Table 4.6.1-25: ULInformationTransfer

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| ULInformationTransfer ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| ulInformationTransfer-r8 SEQUENCE { |  |  |  |  |
| dedicatedInfoType CHOICE { |  |  |  |  |
| dedicatedInfoNAS |  | Present but contents not checked |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	UEAssistanceInformation

Table 4.6.1-26: UEAssistanceInformation

| Derivation Path: 36.331, Clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEAssistanceInformation-r11 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| UEAssistanceInformation-r11-IEs SEQUENCE { |  |  |  |
| powerPreferenceIndication-r11 | lowPowerConsumption |  | Low_Power |
| powerPreferenceIndication-r11 | normal |  | Normal |
| laterNonCriticalExtension | Not Present |  |  |
| nonCriticalExtension SEQUENCE { |  |  | V2X |
| sps-AssistanceInformation-r14 SEQUENCE { |  |  | SL_SPS |
| trafficPatternInfoListSL-r14 SEQUENCE (SIZE (1..maxTrafficPattern-r14)) OF SEQUENCE{ |  |  |  |
| trafficPeriodicity-r14 | Any allowed value |  |  |
| timingOffset-r14 | Any allowed value - |  |  |
| priorityInfoSL-r14 | Any allowed value |  |  |
| logicalChannelIdentityUL-r14 | Any allowed value |  |  |
| messageSize-r14 | Any allowed value |  |  |
| } |  |  |  |
| trafficPatternInfoListUL-r14 | Not Present |  |  |
| } |  |  |  |
| DelayBudgetReport-r14::= CHOICE { |  |  | eVoLTE |
| type1 | ms0 |  | DRX_Cycle |
| type2 | ms0 |  | Coverage |
| } |  |  |  |
| nonCriticalExtension |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Low_Power | The UE indicates that it prefers a configuration primarily optimised for power saving |
| Normal | The UE indicates that it does not prefer a configuration primarily optimised for power saving |
| V2X | V2X Sidelink Communication |
| SL_SPS | Sidelink SPS configuration |
| eVoLTE | The UE Indicates it preferrs adjustment to connected mode DRX or coverage enhancement configuration |
| DRX_Cycle | The UE indicates that it prefers an adjustment in the connected mode DRX cycle length |
| Coverage | The UE indicates that it prefers coverage enhancement configuration change |

#### –	ULInformationTransferMRDC

Table 4.6.1-27: ULInformationTransferMRDC

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| ULInformationTransferMRDC-r15 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| ulInformationTransferMRDC-r15 SEQUENCE { |  |  |  |  |
| ul-DCCH-MessageNR-r15 |  | OCTET STRING including the MeasurementReport message according TS 38.508-1 [67], table 4.6.1-7. |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

### 4.6.2	System information blocks

See subclause 4.4.3.3 in this document.

### 4.6.3	Radio resource control information elements

Table 4.6.3-0A: Void

Table 4.6.3-0B: Void

#### –	BCCH-Config-DEFAULT

Table 4.6.3-1: BCCH-Config-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BCCH-Config-DEFAULT::= SEQUENCE { |  |  |  |
| modificationPeriodCoeff | n4 | To provide reliable delivery of SI change notifications. |  |
| } |  |  |  |

#### –	CellSelectionInfoCE-r13-DEFAULT

Table 4.6.3-1AB: CellSelectionInfoCE-r13-DEFAULT

| Derivation Path: 36.331 clause 6.3.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellSelectionInfoCE-r13::= SEQUENCE { |  |  |  |
| q-RxLevMinCE-r13 | -70 (-140 dBm) | For RF/RRM test cases |  |
|  | -53 (-106 dBm) | For signalling test cases |  |
| q-QualMinRSRQ-CE-r13 | Not present |  |  |
| } |  |  |  |

#### –	CQI-ReportAperiodic-r10-DEFAULT

Table 4.6.3-1A: CQI-ReportAperiodic-r10-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportAperiodic-r10 ::=CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | rm30 |  |  |
|  | rm20 |  | CEmodeA |
|  | FFS |  | CEmodeB |
| aperiodicCSI-Trigger-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| CEmodeA | CE mode A test environment |
| CEmodeB | CE mode B test environment |

#### –	CQI-ReportConfig-DEFAULT

Table 4.6.3-2: CQI-ReportConfig-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic | rm30 |  |  |
|  | rm20 |  | CEmodeA |
|  | FFS |  | CEmodeB |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic | Not present |  |  |
| cqi-ReportPeriodic CHOICE { |  |  | CQI_PERIODIC |
| setup SEQUENCE { |  |  |  |
| cqi-PUCCH-ResourceIndex | 0 |  |  |
| cqi-pmi-ConfigIndex | 25 | (see Table 7.2.2-1A in TS 36.213) | FDD |
|  | 24 | (see Table 7.2.2-1C in TS 36.213) | TDD |
| cqi-FormatIndicatorPeriodic CHOICE { |  |  |  |
| widebandCQI | NULL |  |  |
| } |  |  |  |
| ri-ConfigIndex | 483 | (see Table 7.2.2-1B in TS 36.213) | FDD |
|  | 484 | (see Table 7.2.2-1B in TS 36.213) This value for TDD is selected together with cqi-pmi-ConfigIndex based on TDD configuration 1. | TDD |
| simultaneousAckNackAndCQI | FALSE |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| CQI_PERIODIC | When periodic CQI reporting should be enabled |
| FDD | FDD cell environment |
| TDD | TDD cell environment |
| CEmodeA | CE mode A test environment |
| CEmodeB | CE mode B test environment |

#### –	CQI-ReportConfig-r10-DEFAULT

Table 4.6.3-2AA: CQI-ReportConfig-r10-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-r10 ::= SEQUENCE { |  |  |  |
| cqi-ReportAperiodic-r10 | CQI-ReportAperiodic-r10-DEFAULT |  |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic-r10 | Not present |  |  |
|  | CQI-ReportPeriodic-r10-DEFAULT |  | CQI_PERIODIC |
| pmi-RI-Report-r9 | Not present |  |  |
| csi-SubframePatternConfig-r10 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| CQI_PERIODIC | When periodic CQI reporting should be enabled |

#### –	CQI-ReportConfig-v1130-DEFAULT

Table 4.6.3-2ABA: CQI-ReportConfig-v1130-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-v1130 ::= SEQUENCE { |  |  |  |
| cqi-ReportPeriodic-v1130 SEQUENCE { |  |  |  |
| simultaneousAckNackAndCQI-Format3-r11 | Not present |  |  |
| cqi-ReportPeriodicProcExtToReleaseList-r11 | Not present |  |  |
| cqi-ReportPeriodicProcExtToAddModList-r11 SEQUENCE (SIZE (1..maxCQI-ProcExt-r11)) OF SEQUENCE { | 1 entry |  |  |
| cqi-ReportPeriodicProcExtId-r11[1] SEQUENCE { |  |  |  |
| cqi-ReportPeriodicProcExtId-r11 | 1 |  |  |
| cqi-pmi-ConfigIndex-r11 | Not present |  |  |
| cqi-FormatIndicatorPeriodic-r11 CHOICE { |  |  |  |
| widebandCQI-r11 SEQUENCE { |  |  |  |
| csi-ReportMode-r11 | Not present |  |  |
| } |  |  |  |
| subbandCQI-r11 SEQUENCE { |  |  |  |
| k | Not present |  |  |
| periodicityFactor-r11 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| ri-ConfigIndex-r11 | Not present |  |  |
| csi-ConfigIndex-r11 CHOICE { |  |  |  |
| release | NULL |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-pmi-ConfigIndex2-r11 | Not present |  |  |
| ri-ConfigIndex2-r11 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cqi-ReportBoth-r11 SEQUENCE { |  |  |  |
| csi-IM-ConfigToReleaseList-r11 | Not present |  |  |
| csi-IM-ConfigToAddModList-r11 SEQUENCE (SIZE (1..maxCSI-IM-r11)) OF SEQUENCE { | 1 entry |  |  |
| csi-IM-ConfigId-r11[1] | 1 |  |  |
| resourceConfig-r11[1] | 2 |  | FDD |
|  | 2 |  | TDD |
| subframeConfig-r11[1] | 1 |  | FDD |
|  | 3 |  | TDD |
| } |  |  |  |
| csi-ProcessToReleaseList-r11 | Not present |  |  |
| csi-ProcessToAddModList-r11SEQUENCE (SIZE (1..maxCSI-Proc-r11)) OF SEQUENCE { | 1 entry |  |  |
| csi-ProcessId-r11[1] | 1 |  |  |
| csi-RS-ConfigNZPId-r11[1] | 1 |  |  |
| csi-IM-ConfigId-r11[1] | 1 |  |  |
| p-C-AndCBSRList-r11[1] SEQUENCE (SIZE (1..2)) OF SEQUENCE { | 1 entry |  |  |
| p-C-r11[1] | -6 |  |  |
| codebookSubsetRestriction-r11[1] | 1111111 |  |  |
| } |  |  |  |
| cqi-ReportBothProc-r11[1] SEQUENCE { |  |  |  |
| ri-Ref-CSI-ProcessId-r11 | 1 |  |  |
| pmi-RI-Report-r11 | Not present |  |  |
| } |  |  |  |
| cqi-ReportPeriodicProcId-r11[1] | 1 |  |  |
| cqi-ReportAperiodicProc-r11[1] SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r11 | Not present |  |  |
| trigger01-r11 | Not present |  |  |
| trigger10-r11 | Not present |  |  |
| trigger11-r11 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |

#### –	CQI-ReportConfig-v1130-eIMTA

Table 4.6.3-2ABB: CQI-ReportConfig-v1130-eIMTA

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-v1130-eIMTA ::= SEQUENCE { |  |  |  |
| cqi-ReportPeriodic-v1130 SEQUENCE { |  |  |  |
| simultaneousAckNackAndCQI-Format3-r11 | Not present |  |  |
| cqi-ReportPeriodicProcExtToReleaseList-r11 | Not present |  |  |
| cqi-ReportPeriodicProcExtToAddModList-r11 SEQUENCE (SIZE (1..maxCQI-ProcExt-r11)) OF SEQUENCE { | 1 entry |  |  |
| cqi-ReportPeriodicProcExtId-r11[1] SEQUENCE { |  |  |  |
| cqi-ReportPeriodicProcExtId-r11 | 1 |  |  |
| cqi-pmi-ConfigIndex-r11 | Not present |  |  |
| cqi-FormatIndicatorPeriodic-r11 CHOICE { |  |  |  |
| widebandCQI-r11 SEQUENCE { |  |  |  |
| csi-ReportMode-r11 | Not present |  |  |
| } |  |  |  |
| subbandCQI-r11 SEQUENCE { |  |  |  |
| k | Not present |  |  |
| periodicityFactor-r11 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| ri-ConfigIndex-r11 | Not present |  |  |
| csi-ConfigIndex-r11 CHOICE { |  |  |  |
| release | NULL |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-pmi-ConfigIndex2-r11 | Not present |  |  |
| ri-ConfigIndex2-r11 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cqi-ReportBoth-r11 SEQUENCE { |  |  |  |
| csi-IM-ConfigToReleaseList-r11 | Not present |  |  |
| csi-IM-ConfigToAddModList-r11 SEQUENCE (SIZE (1..maxCSI-IM-r11)) OF SEQUENCE { | 1 entry |  |  |
| csi-IM-ConfigId-r11[1] | 1 |  |  |
| resourceConfig-r11[1] | 2 |  | FDD |
|  | 2 |  | TDD |
| subframeConfig-r11[1] | 1 |  | FDD |
|  | 3 |  | TDD |
| } |  |  |  |
| csi-ProcessToReleaseList-r11 | Not present |  |  |
| csi-ProcessToAddModList-r11SEQUENCE (SIZE (1..maxCSI-Proc-r11)) OF SEQUENCE { | 1 entry |  |  |
| csi-ProcessId-r11[1] | 1 |  |  |
| csi-RS-ConfigNZPId-r11[1] | 1 |  |  |
| csi-IM-ConfigId-r11[1] | 1 |  |  |
| p-C-AndCBSRList-r11[1] SEQUENCE (SIZE (1..2)) OF SEQUENCE { | 1 entry |  |  |
| p-C-r11[1] | -6 |  |  |
| codebookSubsetRestriction-r11[1] | 1111111 |  |  |
| } |  |  |  |
| cqi-ReportBothProc-r11[1] SEQUENCE { |  |  |  |
| ri-Ref-CSI-ProcessId-r11 | 1 |  |  |
| pmi-RI-Report-r11 | Not present |  |  |
| } |  |  |  |
| cqi-ReportPeriodicProcId-r11[1] | 1 |  |  |
| cqi-ReportAperiodicProc-r11[1] SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r11 | Not present |  |  |
| trigger01-r11 | Not present |  |  |
| trigger10-r11 | Not present |  |  |
| trigger11-r11 | Not present |  |  |
| } |  |  |  |
| alternativeCodebookEnabledFor4TXProc-r12 | Not present |  |  |
| csi-IM-ConfigIdList-r12[1] CHOICE { |  |  |  |
| Setup SEQUENCE (SIZE (1..2)) OF{ | 1 entry |  |  |
| CSI-IM-ConfigId-r12[1] | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| cqi-ReportAperiodicProc2-r12[1] CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r11 | Not present |  |  |
| trigger01-r11 | Not present |  |  |
| trigger10-r11 | Not present |  |  |
| trigger11-r11 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |

#### –	CQI-ReportConfig-v1250-DEFAULT

Table 4.6.3-2ACA: CQI-ReportConfig-v1250-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-v1250 | Not present |  | DL 256QAM |
| CQI-ReportConfig-v1250 ::= SEQUENCE { |  |  |  |
| csi-SubframePatternConfig-r12 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| csi-MeasSubframeSets-r12 | 0001100111 |  |  |
| } |  |  |  |
| } |  |  |  |
| cqiCQI-ReportBoth-v1250 | Not present |  | DL 256QAM |
| cqiCQI-ReportBoth-v1250 SEQUENCE { |  |  |  |
| csi-IM-ConfigToReleaseListExt-r12 | Not present |  |  |
| csi-IM-ConfigToAddModListExt-r12 SEQUENCE { | 1 entry |  |  |
| csi-IM-ConfigId-v1250[1] | 1 |  |  |
| resourceConfig-r12[1] | 2 |  | FDD |
|  | 2 |  | TDD |
| subframeConfig-r12[1] | 1 |  | FDD |
|  | 3 |  | TDD |
| } |  |  |  |
| } |  |  |  |
| cqi-ReportAperiodic-v1250 | Not present |  |  |
| altCQI-Table-r12 | Not present |  |  |
|  | allSubframes |  | DL 256QAM |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |
| TM10 | Transmission mode 10 |
| DL 256QAM | When DL 256QAM is used |

#### –	CQI-ReportConfigSCell-r10-DEFAULT

Table 4.6.3-2AB: CQI-ReportConfigSCell-r10-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfigSCell-r10 ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | rm30 |  |  |
| nomPDSCH-RS-EPRE-Offset-r10 | 0 |  |  |
| cqi-ReportPeriodicSCell-r10 | Not present |  |  |
|  | CQI-ReportPeriodic-r10-DEFAULT |  | CQI_PERIODIC |
| pmi-RI-Report-r10 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| CQI_PERIODIC | When periodic CQI reporting should be enabled |

#### –	CQI-ReportPeriodic-r10-DEFAULT

Table 4.6.3-2AC: CQI-ReportPeriodic-r10-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportPeriodic-r10 ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-PUCCH-ResourceIndex-r10 | 0 |  |  |
| cqi-PUCCH-ResourceIndexP1-r10 | Not present |  |  |
| cqi-pmi-ConfigIndex | 25 |  | FDD |
|  | 24 |  | TDD |
| cqi-FormatIndicatorPeriodic-r10 CHOICE { |  |  |  |
| widebandCQI-r10 SEQUENCE { |  |  |  |
| csi-ReportMode-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| ri-ConfigIndex | Not present |  | SCell_Config |
|  | 483 |  | FDD and NOT SCell_Config |
|  | 484 |  | TDD and NOT SCell_Config |
| simultaneousAckNackAndCQI | FALSE |  |  |
| cqi-Mask-r9 | Not present |  |  |
| csi-ConfigIndex-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |
| SCell_Config | CQI-ReportPeriodic configuration for SCell. |

Table 4.6.3-2AD: Void

Table 4.6.3-2AE: Void

#### –	CSI-RS-ConfigNZP-r11-DEFAULT

Table 4.6.3-2AF: CSI-RS-ConfigNZP-r11-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ConfigNZP-r11 ::= SEQUENCE { |  |  |  |
| csi-RS-ConfigNZPId-r11 | 1 |  |  |
| antennaPortsCount-r11 | an2 |  |  |
| resourceConfig-r11 | 0 |  | FDD |
|  | 0 |  | TDD |
| subframeConfig-r11 | 1 |  | FDD |
|  | 3 |  | TDD |
| scramblingIdentity-r11 | 0 |  |  |
| qcl-CRS-Info-r11 SEQUENCE { |  |  |  |
| qcl-ScramblingIdentity-r11 | 0 |  |  |
| crs-PortsCount-r11 | n2 |  |  |
| mbsfn-SubframeConfigList-r11 CHOICE { |  |  |  |
| release | NULL |  |  |
| setup SEQUENCE { |  |  |  |
| subframeConfigList | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |

#### –	CSI-RS-ConfigZP-r11-DEFAULT

Table 4.6.3-2AG: CSI-RS-ConfigZP-r11-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ConfigZP-r11 ::= SEQUENCE { |  |  |  |
| csi-RS-ConfigZPId-r11 | 1 |  |  |
| resourceConfigList-r11 | 0010000000000000 |  |  |
| subframeConfig-r11 | 1 |  | FDD |
|  | 3 |  | TDD |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |

#### –	DMRS-Config-r11-DEFAULT

Table 4.6.3-2AH: DMRS-Config-r11-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMRS-Config-r11 ::= CHOICE { |  |  |  |
| release | NULL |  |  |
| setup SEQUENCE { |  |  |  |
| scramblingIdentity-r11 | 0 |  |  |
| scramblingIdentity2-r11 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	DRB-ToAddModList-RECONFIG

Table 4.6.3-2A: DRB-ToAddModList-RECONFIG

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DRB-ToAddModList ::= SEQUENCE (SIZE (1..maxDRB)) OF SEQUENCE { | 2 Entries |  |  |
| eps-BearerIdentity[1] | Not present |  |  |
| drb-Identity[1] | 1 |  |  |
| pdcp-Config[1] | Not present |  |  |
| rlc-Config[1] | RLC-Config-DRB-AM-RECONFIG |  |  |
| logicalChannelIdentity[1] | Not present |  |  |
| logicalChannelConfig[1] | Not present |  |  |
| eps-BearerIdentity[2] | Not present |  |  |
| drb-Identity[2] | 2 |  |  |
| pdcp-Config[2] | Not present |  |  |
| rlc-Config[2] | RLC-Config-DRB-AM-RECONFIG |  |  |
| logicalChannelIdentity[2] | Not present |  |  |
| logicalChannelConfig[2] | Not present |  |  |
| } |  |  |  |

#### –	EPDCCH-Config-r11-DEFAULT

Table 4.6.3-2B: EPDCCH-Config-r11-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPDCCH-Config-r11 ::= SEQUENCE{ |  |  |  |
| config-r11{ |  |  |  |
| setup SEQUENCE { |  |  |  |
| subframePatternConfig-r11{ } | Not present |  | CEmodeA, CEmodeB |
| subframePatternConfig-r11{ |  |  |  |
| setup SEQUENCE { |  |  |  |
| subframePattern-r11 CHOICE { |  |  |  |
| subframePatternFDD-r10 | 0000000000000000000000000000000000000000 |  | FDD |
|  | 1111111110 1111111101 1111111011 1111110111 |  | FDD AND TM9 |
| subframePatternTDD-r10{ |  |  | TDD |
| subframeConfig1-5-r10 | 0000000000 0000000000 |  |  |
|  | 1100111001 1100111001 |  | TM9 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| startSymbol-r11 | Not present |  |  |
|  | 2 |  | TM9 |
| setConfigToReleaseList-r11 | Not present |  |  |
| setConfigToAddModList-r11 SEQUENCE { | 2 entry |  |  |
| setConfigId-r11[1] | 0 |  |  |
| transmissionType-r11[1] | distributed |  |  |
|  | localized |  | TM10 |
| resourceBlockAssignment-r11[1] SEQUENCE{ |  |  |  |
| numberPRB-Pairs-r11 | n4 |  |  |
|  | n2 |  | TM9 |
|  | n8 |  | TM10 |
| resourceBlockAssignment-r11 | 101001000101101110 |  |  |
|  | 10010011000 |  | TM9 |
|  | 11100100101110110011001101100 |  | TM10 |
| } |  |  |  |
| dmrs-ScramblingSequenceInt-r11[1] | 0 |  |  |
| pucch-ResourceStartOffset-r11[1] | 0 |  |  |
| re-MappingQCL-ConfigListId-r11[1] | Not present |  |  |
|  | 0 |  | TM10 |
| setConfigId-r11[2] | 1 |  |  |
| transmissionType-r11[2] | distributed |  |  |
|  | localized |  | TM9, TM10 |
| resourceBlockAssignment-r11[2] SEQUENCE{ |  |  |  |
| numberPRB-Pairs-r11 | n8 |  |  |
| resourceBlockAssignment-r11 | 11100100101110110011001101100 |  |  |
| } |  |  |  |
| dmrs-ScramblingSequenceInt-r11[2] | 0 |  |  |
| pucch-ResourceStartOffset-r11[2] | 0 |  |  |
| re-MappingQCL-ConfigListId-r11[2] | Not present |  |  |
|  | 0 |  | TM10 |
| } |  |  |  |
| setConfigToAddModList-r11 SEQUENCE { | 1 entry |  | CEmodeA, CEmodeB |
| setConfigId-r11[1] | 0 |  |  |
| transmissionType-r11[1] | distributed |  |  |
| resourceBlockAssignment-r11[1] SEQUENCE{ |  |  |  |
| numberPRB-Pairs-r11 | n2 |  |  |
| resourceBlockAssignment-r11 | 1110 |  | SIG |
|  | 0000 |  | RF |
| } |  |  |  |
| dmrs-ScramblingSequenceInt-r11[1] | 0 |  |  |
| pucch-ResourceStartOffset-r11[1] | 0 |  |  |
| re-MappingQCL-ConfigListId-r11[1] | Not present |  |  |
| numberPRB-Pairs-v1310 CHOICE[1] { |  |  |  |
| Setup | n6 |  |  |
| } |  |  |  |
| mpdcch-config-r13 CHOICE[1] { |  |  |  |
| setup SEQUENCE { |  |  |  |
| csi-NumRepetitionCE-r13 | sf1 |  |  |
| mpdcch-pdsch-HoppingConfig-r13 | off |  |  |
| mpdcch-StartSF-UESS-r13 CHOICE { |  |  |  |
| fdd-r13 | v1 |  | FDD |
| tdd-r13 | v1 |  | TDD |
| } |  |  |  |
| mpdcch-NumRepetition-r13 | r1 | No repetitions to reduce impact on legacy E-UTRA test cases for category M1 UEs. |  |
| mpdcch-Narrowband-r13 | 5 | Narrowband index 4 | RF and 20 MHz BW |
|  | 4 | Narrowband index 3 | RF and (15 MHz BW or 5 MHz BW) |
|  | 7 | Narrowband index 6 | RF and 10 MHz BW |
|  | 2 | Narrowband index 1 | RF and 3 MHz BW |
|  | 1 | Narrowband index 0 | RF and 1.4 MHz BW |
|  | Value as specified in TS 36.523-3 clause 7.3.3.9 |  | SIG |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |
| TM9 | Transmission mode 9 |
| TM10 | Transmission mode 10 |
| CEmodeA | CE mode A test environment |
| CEmodeB | CE mode B test environment |
| RF | Used for RF, performance and RRM testing |
| SIG | Used for protocol testing |
| 20 MHz BW | Used for cells with 20 MHz bandwidth |
| 15 MHz BW | Used for cells with 15 MHz bandwidth |
| 10 MHz BW | Used for cells with 10 MHz bandwidth |
| 5 MHz BW | Used for cells with 5 MHz bandwidth |
| 3 MHz BW | Used for cells with 3 MHz bandwidth |
| 1.4 MHz BW | Used for cells with 1.4 MHz bandwidth |

#### –	EPDCCH-Config-r11-eIMTA

Table 4.6.3-2C: EPDCCH-Config-r11-eIMTA

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPDCCH-Config-r11-eIMTA ::= SEQUENCE{ |  |  |  |
| config-r11{ |  |  |  |
| setup SEQUENCE { |  |  |  |
| subframePatternConfig-r11{ |  |  |  |
| setup SEQUENCE { |  |  |  |
| subframePattern-r11 CHOICE { |  |  |  |
| subframePatternFDD-r10 | 0000000000000000000000000000000000000000 |  | FDD |
|  | 1111111110 1111111101 1111111011 1111110111 |  | FDD, TM9 |
| subframePatternTDD-r10{ |  |  | TDD |
| subframeConfig1-5-r10 | 0000000000 0000000000 |  |  |
|  | 1100111001 1100111001 |  | TM9 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| startSymbol-r11 | Not present |  |  |
|  | 2 |  | TM9 |
| setConfigToReleaseList-r11 | Not present |  |  |
| setConfigToAddModList-r11 SEQUENCE { | 2 entry |  |  |
| setConfigId-r11[1] | 0 |  |  |
| transmissionType-r11[1] | distributed |  |  |
|  | localized |  | TM10 |
| resourceBlockAssignment-r11[1] SEQUENCE{ |  |  |  |
| numberPRB-Pairs-r11 | n4 |  |  |
|  | n2 |  | TM9 |
|  | n8 |  | TM10 |
| resourceBlockAssignment-r11 | 101001000101101110 |  |  |
|  | 10010011000 |  | TM9 |
|  | 11100100101110110011001101100 |  | TM10 |
| } |  |  |  |
| dmrs-ScramblingSequenceInt-r11[1] | 0 |  |  |
| pucch-ResourceStartOffset-r11[1] | 0 |  |  |
| re-MappingQCL-ConfigListId-r11[1] | Not present |  |  |
|  | 0 |  | TM10 |
| csi-RS-ConfigZPId2-r12[1] | Not present |  |  |
| csi-RS-ConfigZPId2-r12[1] CHOICE { |  |  |  |
| Setup { |  |  |  |
| CSI-RS-ConfigZPId-r11[1] | 1 |  | TM10 |
| } |  |  |  |
| } |  |  |  |
| setConfigId-r11[2] | 1 |  |  |
| transmissionType-r11[2] | distributed |  |  |
|  | localized |  | TM9, TM10 |
| resourceBlockAssignment-r11[2] SEQUENCE{ |  |  |  |
| numberPRB-Pairs-r11 | n8 |  |  |
| resourceBlockAssignment-r11 | 11100100101110110011001101100 |  |  |
| } |  |  |  |
| dmrs-ScramblingSequenceInt-r11[2] | 0 |  |  |
| pucch-ResourceStartOffset-r11[2] | 0 |  |  |
| re-MappingQCL-ConfigListId-r11[2] | Not present |  |  |
|  | 0 |  | TM10 |
| csi-RS-ConfigZPId2-r12[2] | Not present |  |  |
| csi-RS-ConfigZPId2-r12[2] CHOICE { |  |  |  |
| Setup { |  |  |  |
| CSI-RS-ConfigZPId-r11[2] | 1 |  | TM10 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |
| TM9 | Transmission mode 9 |
| TM10 | Transmission mode 10 |

#### –	FreqHoppingParameters-r13-DEFAULT

Table 4.6.3-2D: FreqHoppingParameters-r13-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FreqHoppingParameters-r13-DEFAULT::= SEQUENCE { |  |  |  |
| mpdcch-pdsch-HoppingNB-r13 | Not present |  |  |
| interval-DLHoppingConfigCommonModeA-r13 CHOICE {} | Not present |  |  |
| interval-DLHoppingConfigCommonModeB-r13 CHOICE { | Not present |  |  |
| interval-ULHoppingConfigCommonModeA-r13 CHOICE { |  |  |  |
| interval-FDD-r13 | int4 |  | FDD |
| interval-TDD-r13 | int5 |  | TDD |
| } |  |  |  |
| interval-ULHoppingConfigCommonModeB-r13 CHOICE { |  |  |  |
| interval-FDD-r13 | int16 |  | FDD |
| interval-TDD-r13 | int20 |  | TDD |
| } |  |  |  |
| mpdcch-pdsch-HoppingOffset-r13 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |

#### –	HighSpeedConfig-r14-DEFAULT

Table 4.6.3-2E: HighSpeedConfig-r14-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| HighSpeedConfig-r14-DEFAULT::= SEQUENCE { |  |  |  |
| highSpeedEnhancedMeasFlag-r14 | true |  |  |
| highSpeedEnhancedDemodulationFlag-r14 | true |  |  |
| } |  |  |  |

#### –	HighSpeedConfigSCell-r14-DEFAULT

Table 4.6.3-2F: HighSpeedConfigSCell-r14-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| HighSpeedConfig-r14-DEFAULT::= SEQUENCE { |  |  |  |
| highSpeedEnhancedDemodulationFlag-r14 | true |  |  |
| } |  |  |  |

#### –	PCCH-Config-DEFAULT

Table 4.6.3-3: PCCH-Config-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PCCH-Config-DEFAULT::= SEQUENCE { |  |  |  |
| defaultPagingCycle | rf128 | Typical value in real network. |  |
| nB | oneT |  |  |
| } |  |  |  |

#### –	PCCH-Config-v1310-DEFAULT

Table 4.6.3-3A: PCCH-Config-v1310-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PCCH-Config-v1310DEFAULT::= SEQUENCE { |  |  |  |
| paging-narrowBands-r13 | 1 |  |  |
| mpdcch-NumRepetition-Paging-r13 | r16 |  |  |
| nB-v1310 | Not present | Not present means that nB value in PDCCH-Config applies. |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| RF | Used for RF, performance and RRM testing |
| SIG | Used for protocol testing |

#### –	PHICH-Config-DEFAULT

Table 4.6.3-4: PHICH-Config-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PHICH-Config-DEFAULT ::= SEQUENCE { |  |  |  |
| phich-Duration | normal |  |  |
| phich-Resource | one | Ng=1 has been selected to ensure enough PHICH resources from the real network point of view. |  |
| } |  |  |  |

#### –	PDSCH-ConfigCommon-DEFAULT

Table 4.6.3-5: PDSCH-ConfigCommon-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ConfigCommon-DEFAULT ::= SEQUENCE { |  |  |  |
| referenceSignalPower | 18 (dBm) |  |  |
| p-b | 0 ( = 1) |  | 1TX |
|  | 1 ( = 1) |  | 2TX, 4TX |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| 1TX | 1 SS Tx antenna environment |
| 2TX | 2 SS Tx antenna environment |
| 4TX | 4 SS Tx antenna environment |

#### –	PDSCH-ConfigCommon-v1310-DEFAULT

Table 4.6.3-5A: PDSCH-ConfigCommon-v1310-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ConfigCommon-v1310-DEFAULT ::= SEQUENCE { |  |  |  |
| pdsch-maxNumRepetitionCEmodeA-r13 | r16 | Minimum value | CEmodeA AND SIG |
|  | [r32] | Maximium value | CEmodeA AND RF |
| pdsch-maxNumRepetitionCEmodeB-r13 | r192 | Minimum value | CEmodeB AND SIG |
|  | [2048] | Maximium value | CEmodeB AND RF |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| CEmodeA | CE mode A test environment |
| CEmodeB | CE mode B test environment |
| RF | Value used for RF, performance and RRM testing |
| SIG | Value used for protocol testing |

#### –	PDSCH-ConfigDedicated-DEFAULT

Table 4.6.3-6: PDSCH-ConfigDedicated-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| p-a | dB-3 for signalling test cases; dB0 for others | dB-3 for signalling test cases to reduce interference from PDSCH of intra-frequency neighbour cells | 1TX |
|  | dB-3 |  | 2TX, 4TX |
| } |  |  |  |

#### –	PDSCH-ConfigDedicated-v1130-DEFAULT

Table 4.6.3-6AA: PDSCH-ConfigDedicated-v1130-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ConfigDedicated-v1130 ::= SEQUENCE { |  |  |  |
| dmrs-ConfigPDSCH-r11 | DMRS-Config-r11-DEFAULT |  |  |
| qcl-Operation | typeB |  |  |
| re-MappingQCLConfigToReleaseList-r11 | Not present |  |  |
| re-MappingQCLConfigToAddModList-r11 SEQUENCE (SIZE (1..maxRE-MapQCL-r11)) OF SEQUENCE { | 1 entry |  |  |
| pdsch-RE-MappingQCL-ConfigId-r11[1] | 1 |  |  |
|  | 0 |  | TM10 |
| optionalSetOfFields-r11[1] SEQUENCE { |  |  |  |
| crs-PortsCount-r11 | Not present |  |  |
| crs-FreqShift-r11 | Not present |  |  |
| mbsfn-SubframeConfigList-r11{ |  |  |  |
| setup SEQUENCE { |  |  |  |
| subframeConfigList | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| pdsch-Start-r11 | Not present |  |  |
|  | 2 |  | [TM10 |
| } |  |  |  |
| csi-RS-ConfigZPId-r11[1] | 1 |  |  |
| qcl-CSI-RS-ConfigNZPId-r11[1] | 1 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| TM10 | Transmission mode 10 |

#### –	PhysicalConfigDedicatedSCell-r10-DEFAULT

Table 4.6.3-6A: PhysicalConfigDedicatedSCell-r10-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicatedSCell-r10-DEFAULT ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| antennaInfo-r10 SEQUENCE { |  |  |  |
| transmissionMode-r10 | tm1 |  | 1TX |
|  | tm2 |  | 2TX |
| codebookSubsetRestriction-r10 | Not present |  |  |
| ue-TransmitAntennaSelection | release |  |  |
| } |  |  |  |
| crossCarrierSchedulingConfig-r10 | Not present |  |  |
| csi-RS-Config-r10 | Not present |  |  |
| pdsch-ConfigDedicated-r10 | PDSCH-ConfigDedicated-DEFAULT |  |  |
| } |  |  |  |
| ul-Configuration-r10 | Not present |  |  |
| ul-Configuration-r10 SEQUENCE { |  |  | UL CA |
| antennaInfoUL-r10 | Not present |  |  |
| pusch-ConfigDedicatedSCell-r10 | Not present |  |  |
| uplinkPowerControlDedicatedSCell-r10 | UplinkPowerControlDedicatedSCell-r10-DEFAULT |  |  |
| cqi-ReportConfigSCell-r10 | CQI-ReportConfigSCell-r10-DEFAULT |  |  |
| soundingRS-UL-ConfigDedicated-r10 | Not present |  |  |
| soundingRS-UL-ConfigDedicated-v1020 | Not present |  |  |
| soundingRS-UL-ConfigDedicatedAperiodic-r10 | Not present |  |  |
| } |  |  |  |
| cqi-ReportConfigSCell-v1250 | Not present |  |  |
|  | CQI-ReportConfig-v1250-DEFAULT |  | DL 256QAM |
| pucch-Cell-r13 | Not present |  |  |
|  | true |  | PUCCH-SCell1 |
| pucch-SCell | Not present |  |  |
| pucch-SCell CHOICE { |  |  | PUCCH-SCell |
| setup SEQUENCE { |  |  |  |
| pucch-ConfigDedicated-r13 | PUCCH-ConfigDedicated-r13-DEFAULT |  |  |
| schedulingRequestConfig-r13 | SchedulingRequestConfigSCell-r13-DEFAULT |  |  |
| tpc-PDCCH-ConfigPUCCH-SCell-r13 | TPC-PDCCH-ConfigSCell-r13-DEFAULT |  |  |
| pusch-ConfigDedicated-r13 | Not present |  |  |
| uplinkPowerControlDedicated-r13 | UplinkPowerControlDedicatedSCell-v1310-DEFAULT |  |  |
| } |  |  |  |
| } |  |  |  |
| crossCarrierSchedulingConfig-r13 | Not present |  |  |
| pdcch-ConfigSCell-r13 | Not present |  |  |
| cqi-ReportConfig-v1310 | Not present |  |  |
| pdsch-ConfigDedicated-v1310 | Not present |  |  |
| soundingRS-UL-ConfigDedicated-v1310 | Not present |  |  |
| soundingRS-UL-ConfigDedicatedUpPTsExt-r13 | Not present |  |  |
| soundingRS-UL-ConfigDedicatedAperiodic-v1310 | Not present |  |  |
| soundingRS-UL-ConfigDedicatedAperiodicUpPTsExt-r13 | Not present |  |  |
| csi-RS-Config-v1310 | Not present |  |  |
| Laa-ScellConfiguration-r13 | Not present |  |  |
| Laa-ScellConfiguration-r13 SEQUENCE { |  |  | LAA |
| subframeStartPosition | ‘s0’ |  |  |
| Laa-SCellSubframeConfig | ‘00000000’ |  |  |
| } |  |  |  |
| csi-RS-ConfigNZPToAddModListExt-r13 | Not present |  |  |
| csi-RS-ConfigNZPToReleaseListExt-r13 | Not present |  |  |
| laa-SCellConfiguration-v1430 SEQUENCE { |  |  | eLAA |
| pdcch-ConfigLAA-r14 SEQUENCE { |  |  |  |
| maxNumberOfSchedSubframes-Format0B-r14 | sf4 |  |  |
| maxNumberOfSchedSubframes-Format4B-r14 | sf4 |  |  |
| } |  |  |  |
| } |  |  |  |
| typeB-SRS-TPC-PDCCH-Config-r14 | SRS-TPC-PDCCH-Config-r14-DEFAULT condition SETUP |  | SRS-Switching |
| uplinkPUSCH-LessPowerControlDedicated-v14xy SEQUENCE { |  |  | SRS-Switching |
| p0-UE-PeriodicSRS | 0 |  |  |
| p0-UE-AperiodicSRS | 0 |  |  |
| accumulationEnabled | true |  |  |
| } |  |  |  |
| soundingRS-UL-PeriodicConfigDedicatedList-r14 SEQUENCE (SIZE (1..2)) OF SEQUENCE { |  |  | SRS-Switching |
| soundingRS-UL-ConfigDedicated | SoundingRS-Ul-ConfigDedicated-DEFAULT |  |  |
| } |  |  |  |
| soundingRS-UL-PeriodicConfigDedicatedUpPTsExtList-r14 | Not Present |  |  |
| soundingRS-UL-AperiodicConfigDedicatedList-r14 | Not Present |  |  |
| soundingRS-UL-ConfigDedicatedApUpPTsExtList-r14 | Not Present |  |  |
| uplinkPowerControlDedicatedSCell-v1530 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UL CA | When UL Carrier Aggregation is used |
| 1TX | 1 SS Tx antenna environment |
| 2TX | 2 SS Tx antenna environment |
| DL 256QAM | When DL 256QAM is used |
| LAA | For LAA test cases |
| SRS-Switching | SRS transmission is switched to a PUSCH-less TDD SCell |
| PUCCH-SCell1 | When PUCCH feedback of this SCell is sent on the PUCCH SCell |
| PUCCH-SCell | When this SCell is configured as PUCCH SCell |
| eLAA | When this SCell is configured as uplink LAA SCell |
| Aerial UE | For Aerial vehicles |

#### –	PhysicalConfigDedicatedSCell-r10-eIMTA

Table 4.6.3-6B: PhysicalConfigDedicatedSCell-r10-eIMTA

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicatedSCell-r10-eIMTA ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| antennaInfo-r10 SEQUENCE { |  |  |  |
| transmissionMode-r10 | tm1 |  | 1TX |
|  | tm2 |  | 2TX |
| codebookSubsetRestriction-r10 | Not present |  |  |
| ue-TransmitAntennaSelection | release |  |  |
| } |  |  |  |
| crossCarrierSchedulingConfig-r10 | Not present |  |  |
| csi-RS-Config-r10 | Not present |  |  |
| pdsch-ConfigDedicated-r10 | PDSCH-ConfigDedicated-DEFAULT |  |  |
| } |  |  |  |
| ul-Configuration-r10 | Not present |  |  |
| ul-Configuration-r10 SEQUENCE { |  |  | UL CA |
| antennaInfoUL-r10 | Not present |  |  |
| pusch-ConfigDedicatedSCell-r10 | Not present |  |  |
| uplinkPowerControlDedicatedSCell-r10 | UplinkPowerControlDedicatedSCell-r10-DEFAULT |  |  |
| cqi-ReportConfigSCell-r10 | CQI-ReportConfigSCell-r10-DEFAULT |  |  |
| soundingRS-UL-ConfigDedicated-r10 | Not present |  |  |
| soundingRS-UL-ConfigDedicated-v1020 | Not present |  |  |
| soundingRS-UL-ConfigDedicatedAperiodic-r10 | Not present |  |  |
| } |  |  |  |
| antennaInfo-v1250 | Not present |  | eIMTA |
| eimta-MainConfigSCell-r12 | EIMTA-MainConfigServCell-r12-DEFAULT |  | eIMTA |
|  | Not present |  |  |
| cqi-ReportConfigSCell-v1250 | CQI-ReportConfig-v1250-DEFAULT |  | eIMTA |
|  | Not present |  |  |
| uplinkPowerControlDedicatedSCell-v1250 | UplinkPowerControlDedicated-v1250-DEFAULT |  | eIMTA |
|  | Not present |  |  |
| csi-RS-Config-v1250 | Not present |  | eIMTA |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UL CA | When UL Carrier Aggregation is used. |
| 1TX | 1 SS Tx antenna environment |
| 2TX | 2 SS Tx antenna environment |
| eIMTA | eIMTA cell environment |

#### –	PRACH-Config-DEFAULT

Table 4.6.3-7: PRACH-Config-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PRACH-Config-DEFAULT ::= SEQUENCE { |  |  |  |
| rootSequenceIndex | See clause 4.4.2, Table 4.4.2-1A and clause 6.3.2.2 Table 6.3.2.2-2 | See table 5.7.2-4 in TS 36.211for PRACH format 0-3 | FDD |
|  | See clause 4.4.2, Table 4.4.2-1A and clause 6.3.2.2 Table 6.3.2.2-2 | See table 5.7.2-5 in TS 36.211 for PRACH format 4 | TDD |
| prach-ConfigInfo SEQUENCE { |  |  |  |
| prach-ConfigIndex | 3 | Typical value in real network for FDD (see table 5.7.1-1 and 5.7.1-2 in TS 36.211) | FDD |
|  | 51 | Typical value in real network for TDD (see table 5.7.1-3 and 5.7.1-4 in TS 36.211) | TDD |
| highSpeedFlag | FALSE (Unrestricted set) | High speed train configuration doesn’t apply by default. |  |
| zeroCorrelationZoneConfig | 5 (Ncs configuration = 5) | Assuming cell radius is up to approximately 3 km. | FDD |
|  | 4 (Ncs configuration = 4) | Assuming cell radius is up to approximately 1 km. | TDD |
| prach-FreqOffset | See subclause 4.6.8 | Channel-bandwidth-dependent parameter |  |
| } |  |  |  |
| } |  |  |  |
| Note: Although PRACH format 0-3 is applicable for both FDD and TDD, the PRACH format 4 is used as default for testing for TDD. |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |

#### -	PRACH-Config-v1310-DEFAULT

Table 4.6.3-7A: PRACH-Config-v1310-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PRACH-Config-v1310-DEFAULT ::= SEQUENCE { |  |  |  |
| rsrp-ThresholdsPrachInfoList-r13 SEQUENCE (SIZE(1..3)) OF {} | Not present |  |  |
| rsrp-ThresholdsPrachInfoList-r13 SEQUENCE (SIZE(1..3)) OF { | 1 entry | Threshold values to determine the CE level for PRACH | CEmodeA |
| RSRP-Range[1] | 42 | -99dBm |  |
| } |  |  |  |
| mpdcch-startSF-CSS-RA-r13 CHOICE { |  |  |  |
| fdd-r13 | v1 |  | FDD |
| tdd-r13 | v1 | ENUMERATED { v1, v2, v4, v5, v8, v10, v20, spare } | TDD |
| } |  |  |  |
| prach-HoppingOffset-r13 | Not present |  |  |
| prach-ParametersListCE-r13 SEQUENCE (SIZE(1..maxCE-Level-r13)) OF SEQUENCE { | 2 entries | 1: CE level 0 2: CE level 1 | CEmodeA |
| PRACH-ParametersCE-r13[1] SEQUENCE { |  |  |  |
| prach-ConfigIndex-r13[1] | 3 |  | RF |
|  | 4 |  | SIG_FDD OR RRM_FDD |
|  | 53 |  | SIG_TDD OR RRM_TDD |
| prach-FreqOffset-r13[1] | See subclause 4.6.8 | Same as prach-FrequencyOffset |  |
| prach-StartingSubframe-r13[1] | sf2 |  |  |
| maxNumPreambleAttemptCE-r13[1] | n5 |  |  |
| numRepetitionPerPreambleAttempt-r13[1] | n1 |  |  |
| mpdcch-NarrowbandsToMonitor-r13 SEQUENCE (SIZE(1..2)) OF INTEGER (1..maxAvailNarrowBands-r13)[1] { | 1 entry |  |  |
| mpdcch-NarrowbandsToMonitor-r13[1] | 1 |  | RF OR RRM |
|  | Value as specified in TS 36.523-3 clause 7.3.3.9 |  | SIG |
| } |  |  |  |
| mpdcch-NumRepetition-RA-r13[1] | r1 |  |  |
| prach-HoppingConfig-r13[1] | off |  |  |
| } |  |  |  |
| PRACH-ParametersCE-r13[2] SEQUENCE { |  |  |  |
| prach-ConfigIndex-r13[2] | 3 |  | RF |
|  | 4 |  | SIG_FDD OR RRM_FDD |
|  | 53 |  | SIG_TDD OR RRM_TDD |
| prach-FreqOffset-r13[2] | See subclause 4.6.8 |  |  |
| prach-StartingSubframe-r13[2] | sf128 |  |  |
| maxNumPreambleAttemptCE-r13[2] | n5 |  |  |
| numRepetitionPerPreambleAttempt-r13[2] | n128 |  |  |
| mpdcch-NarrowbandsToMonitor-r13 SEQUENCE (SIZE(1..2)) OF INTEGER (1..maxAvailNarrowBands-r13)[2] { | 1 entry |  |  |
| mpdcch-NarrowbandsToMonitor-r13[1] | 1 |  | RF OR RRM |
|  | Value as specified in TS 36.523-3 clause 7.3.3.9 |  | SIG |
| } |  |  |  |
| mpdcch-NumRepetition-RA-r13[2] | r4 |  |  |
| prach-HoppingConfig-r13[2] | off |  |  |
| } |  |  |  |
| } |  |  |  |
| initial-CE-level-r13 | 0 |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |
| RF | Used for RF, performance and RRM testing |
| RRM | Used for RRM testing |
| RRM_FDD | Used for RRM testing in FDD cell environment |
| RRM_TDD | Used for RRM testing in TDD cell environment |
| SIG | Used for protocol testing |
| SIG_FDD | Used for protocol testing in FDD cell environment |
| SIG_TDD | Used for protocol testing in TDD cell environment |
| CEmodeA | Used for CE mode A testing |

#### –	PRACH-Config-v1430-DEFAULT

Table 4.6.3-7B: PRACH-Config-v1430-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PRACH-Config-v1430-DEFAULT ::= SEQUENCE { |  |  |  |
| rootSequenceIndexHighSpeed-r14 | See clause 4.4.2, Table 4.4.2-1A and clause 6.3.2.2 Table 6.3.2.2-2 | See table 5.7.2-4 in TS 36.211for PRACH format 0-3 | FDD |
|  | See clause 4.4.2, Table 4.4.2-1A and clause 6.3.2.2 Table 6.3.2.2-2 | See table 5.7.2-4 in TS 36.211 for PRACH format 0-3 | TDD |
| prach-ConfigIndexHighSpeed-r14 | 3 | Typical value in real network for FDD (see table 5.7.1-1 and 5.7.1-2 in TS 36.211) | FDD |
|  | 33 | Typical value in real network for TDD (see table 5.7.1-3 and 5.7.1-4 in TS 36.211) | TDD |
| zeroCorrelationZoneConfigHighSpeed-r14 | 11 (Ncs configuration = 118) | Restricted set type B is used | FDD |
|  | 11 (Ncs configuration = 118) | Restricted set type B is used | TDD |
| prach-FreqOffsetHighSpeed-r14 | See subclause 4.6.8 | Channel-bandwidth-dependent parameter |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |

#### -	PRACH-ConfigSIB-DEFAULT

Table 4.6.3-7a: PRACH-ConfigSIB-DEFAULT

Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-7 PRACH-Config-DEFAULT

#### -	PRACH-ConfigSIB-v1310-DEFAULT

Table 4.6.3-7b: PRACH-ConfigSIB-v1310-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PRACH-ConfigSIB-v1310-DEFAULT ::= SEQUENCE { |  |  |  |
| rsrp-ThresholdsPrachInfoList-r13 SEQUENCE (SIZE(1..3)) OF { | 1 entry | Threshold values to determine the CE level 1 for PRACH. RSRP-Range values according to mapping table in TS 36.133 Table 9.1.14.3.3-1. | CEmodeA PRACH_2CE |
| RSRP-Range[1] | 40 | -101dBm |  |
| } |  |  |  |
| rsrp-ThresholdsPrachInfoList-r13 SEQUENCE (SIZE(1..3)) OF { | 2 entries | Threshold values to determine the CE level 1, 2 and 3 for PRACH. RSRP-Range values according to mapping table in TS 36.133 Table 9.1.14.3.3-1. | CEmodeB PRACH_3CE |
| RSRP-Range[1] | 42 | -99 dBm |  |
| RSRP-Range[2] | 34 | -107 dBm |  |
| } |  |  |  |
| rsrp-ThresholdsPrachInfoList-r13 SEQUENCE (SIZE(1..3)) OF { | 3 entries | Threshold values to determine the CE level 1, 2 and 3 for PRACH. RSRP-Range values according to mapping table in TS 36.133 Table 9.1.14.3.3-1. | CEmodeB PRACH_4CE |
| RSRP-Range[1] | 49 | -92 dBm |  |
| RSRP-Range[2] | 42 | -99 dBm |  |
| RSRP-Range[3] | 34 | -107 dBm |  |
| } |  |  |  |
| mpdcch-startSF-CSS-RA-r13 CHOICE { |  |  |  |
| fdd-r13 | v1 |  | FDD |
| tdd-r13 | v2 |  | TDD |
| } |  |  |  |
| prach-HoppingOffset-r13 | Not present |  |  |
| prach-ParametersListCE-r13 SEQUENCE (SIZE(1..maxCE-Level-r13)) OF SEQUENCE { | 2 entries | 1: CE level 0 2: CE level 1 | CEmodeA PRACH_2CE |
| prach-ConfigIndex-r13[1] | 3 |  | RF |
|  | 4 |  | SIG_FDD OR RRM_FDD |
|  | 53 |  | SIG_TDD OR RRM_TDD |
| prach-FreqOffset-r13[1] | See subclause 4.6.8 | Same as prach-FrequencyOffset |  |
| prach-StartingSubframe-r13[1] | sf2 |  |  |
| maxNumPreambleAttemptCE-r13[1] | n5 |  |  |
| numRepetitionPerPreambleAttempt-r13[1] | n1 |  |  |
| mpdcch-NarrowbandsToMonitor-r13 SEQUENCE (SIZE(1..2)) OF INTEGER (1..maxAvailNarrowBands-r13)[1] | 1 entry |  |  |
| mpdcch-NarrowbandsToMonitor-r13[1] | 1 |  | RF |
|  | Value as specified in TS 36.523-3 clause 7.3.3.9 |  | SIG |
| mpdcch-NumRepetition-RA-r13[1] | r1 |  |  |
| prach-HoppingConfig-r13[1] | off |  |  |
| prach-ConfigIndex-r13[2] | 3 |  | RF |
|  | 4 |  | SIG_FDD OR RRM_FDD |
|  | 53 |  | SIG_TDD OR RRM_TDD |
| prach-FreqOffset-r13[2] | See subclause 4.6.8 |  |  |
| prach-StartingSubframe-r13[2] | sf128 |  |  |
| maxNumPreambleAttemptCE-r13[2] | n5 |  |  |
| numRepetitionPerPreambleAttempt-r13[2] | n128 |  |  |
| mpdcch-NarrowbandsToMonitor-r13 SEQUENCE (SIZE(1..2)) OF INTEGER (1..maxAvailNarrowBands-r13)[2] | 1 entry |  |  |
| mpdcch-NarrowbandsToMonitor-r13[1] | 1 |  | RF |
|  | Value as specified in TS 36.523-3 clause 7.3.3.9 |  | SIG |
| mpdcch-NumRepetition-RA-r13[2] | r4 |  |  |
| prach-HoppingConfig-r13[2] | off |  |  |
| } |  |  |  |
| prach-ParametersListCE-r13 SEQUENCE (SIZE(1..maxCE-Level-r13)) OF SEQUENCE { | 3 entries | 1: CE level 0 2: CE level 1 3: CE level 2 | PRACH_3CE |
| prach-ConfigIndex-r13[1] | 3 |  | RF |
|  | 4 |  | SIG_FDD OR RRM_FDD |
|  | 53 |  | SIG_TDD OR RRM_TDD |
| prach-FreqOffset-r13[1] | See subclause 4.6.8 |  |  |
| prach-StartingSubframe-r13[1] | sf2 |  |  |
| maxNumPreambleAttemptCE-r13[1] | n5 |  |  |
| numRepetitionPerPreambleAttempt-r13[1] | n1 |  |  |
| mpdcch-NarrowbandsToMonitor-r13 SEQUENCE (SIZE(1..2)) OF INTEGER (1..maxAvailNarrowBands-r13)[1] | 1 entry |  |  |
| mpdcch-NarrowbandsToMonitor-r13[1] | 1 |  | RF |
|  | Value as specified in TS 36.523-3 clause 7.3.3.9 |  | SIG |
| mpdcch-NumRepetition-RA-r13[1] | r1 |  |  |
| prach-HoppingConfig-r13[1] | Off |  |  |
| prach-ConfigIndex-r13[2] | 3 |  | RF |
|  | 4 |  | SIG_FDD OR RRM_FDD |
|  | 53 |  | SIG_TDD OR RRM_TDD |
| prach-FreqOffset-r13[2] | See subclause 4.6.8 |  |  |
| prach-StartingSubframe-r13[2] | Sf64 |  |  |
| maxNumPreambleAttemptCE-r13[2] | n5 |  |  |
| numRepetitionPerPreambleAttempt-r13[2] | n64 |  |  |
| mpdcch-NarrowbandsToMonitor-r13 SEQUENCE (SIZE(1..2)) OF INTEGER (1..maxAvailNarrowBands-r13)[2] | 1 entry |  |  |
| mpdcch-NarrowbandsToMonitor-r13[1] | 1 |  | RF |
|  | Value as specified in TS 36.523-3 clause 7.3.3.9 |  | SIG |
| mpdcch-NumRepetition-RA-r13[2] | r16 |  |  |
| prach-HoppingConfig-r13[2] | Off |  |  |
| prach-ConfigIndex-r13[3] | 3 |  | RF |
|  | 4 |  | SIG_FDD OR RRM_FDD |
|  | 53 |  | SIG_TDD OR RRM_TDD |
| prach-FreqOffset-r13[3] | See subclause 4.6.8 |  |  |
| prach-StartingSubframe-r13[3] | sf128 |  |  |
| maxNumPreambleAttemptCE-r13[3] | n5 |  |  |
| numRepetitionPerPreambleAttempt-r13[3] | n128 |  |  |
| mpdcch-NarrowbandsToMonitor-r13 SEQUENCE (SIZE(1..2)) OF INTEGER (1..maxAvailNarrowBands-r13)[3] | 1 entry |  |  |
| mpdcch-NarrowbandsToMonitor-r13[1] | 1 |  | RF |
|  | Value as specified in TS 36.523-3 clause 7.3.3.9 |  | SIG |
| mpdcch-NumRepetition-RA-r13[3] | r64 |  |  |
| prach-HoppingConfig-r13[3] | Off |  |  |
| } |  |  |  |
| prach-ParametersListCE-r13 SEQUENCE (SIZE(1..maxCE-Level-r13)) OF SEQUENCE { | 4 entries | 1: CE level 0 2: CE level 1 3: CE level 2 4: CE level 3 | CEmodeB PRACH_4CE |
| prach-ConfigIndex-r13[1] | 3 |  | RF |
|  | 4 |  | SIG_FDD OR RRM_FDD |
|  | 53 |  | SIG_TDD OR RRM_TDD |
| prach-FreqOffset-r13[1] | See subclause 4.6.8 |  |  |
| prach-StartingSubframe-r13[1] | sf2 |  |  |
| maxNumPreambleAttemptCE-r13[1] | n5 |  |  |
| numRepetitionPerPreambleAttempt-r13[1] | n1 |  |  |
| mpdcch-NarrowbandsToMonitor-r13 SEQUENCE (SIZE(1..2)) OF INTEGER (1..maxAvailNarrowBands-r13)[1] | 1 entry |  |  |
| mpdcch-NarrowbandsToMonitor-r13[1] | 1 |  | RF |
|  | Value as specified in TS 36.523-3 clause 7.3.3.9 |  | SIG |
| mpdcch-NumRepetition-RA-r13[1] | r1 |  |  |
| prach-HoppingConfig-r13[1] | Off |  |  |
| prach-ConfigIndex-r13[2] | 3 |  | RF |
|  | 4 |  | SIG_FDD OR RRM_FDD |
|  | 53 |  | SIG_TDD OR RRM_TDD |
| prach-FreqOffset-r13[2] | See subclause 4.6.8 |  |  |
| prach-StartingSubframe-r13[2] | sf32 |  |  |
| maxNumPreambleAttemptCE-r13[2] | n5 |  |  |
| numRepetitionPerPreambleAttempt-r13[2] | n32 |  |  |
| mpdcch-NarrowbandsToMonitor-r13 SEQUENCE (SIZE(1..2)) OF INTEGER (1..maxAvailNarrowBands-r13)[2] | 1 entry |  |  |
| mpdcch-NarrowbandsToMonitor-r13[1] | 1 |  | RF |
|  | Value as specified in TS 36.523-3 clause 7.3.3.9 |  | SIG |
| mpdcch-NumRepetition-RA-r13[2] | r4 |  |  |
| prach-HoppingConfig-r13[2] | Off |  |  |
| prach-ConfigIndex-r13[3] | 3 |  | RF |
|  | 4 |  | SIG_FDD OR RRM_FDD |
|  | 53 |  | SIG_TDD OR RRM_TDD |
| prach-FreqOffset-r13[3] | See subclause 4.6.8 |  |  |
| prach-StartingSubframe-r13[3] | sf64 |  |  |
| maxNumPreambleAttemptCE-r13[3] | n5 |  |  |
| numRepetitionPerPreambleAttempt-r13[3] | n64 |  |  |
| mpdcch-NarrowbandsToMonitor-r13 SEQUENCE (SIZE(1..2)) OF INTEGER (1..maxAvailNarrowBands-r13)[3] | 1 entry |  |  |
| mpdcch-NarrowbandsToMonitor-r13[1] | 1 |  | RF |
|  | Value as specified in TS 36.523-3 clause 7.3.3.9 |  | SIG |
| mpdcch-NumRepetition-RA-r13[3] | r16 |  |  |
| prach-HoppingConfig-r13[3] | Off |  |  |
| prach-ConfigIndex-r13[4] | 3 |  | RF |
|  | 4 |  | SIG_FDD OR RRM_FDD |
|  | 53 |  | SIG_TDD OR RRM_TDD |
| prach-FreqOffset-r13[4] | See subclause 4.6.8 |  |  |
| prach-StartingSubframe-r13[4] | sf128 |  |  |
| maxNumPreambleAttemptCE-r13[4] | n5 |  |  |
| numRepetitionPerPreambleAttempt-r13[4] | n128 |  |  |
| mpdcch-NarrowbandsToMonitor-r13 SEQUENCE (SIZE(1..2)) OF INTEGER (1..maxAvailNarrowBands-r13)[4] | 1 entry |  |  |
| mpdcch-NarrowbandsToMonitor-r13[1] | 1 |  | RF |
|  | Value as specified in TS 36.523-3 clause 7.3.3.9 |  | SIG |
| mpdcch-NumRepetition-RA-r13[4] | r64 |  |  |
| prach-HoppingConfig-r13[4] | Off |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |
| RF | Used for RF, performance and RRM testing |
| RRM | Used for RRM testing |
| RRM_FDD | Used for RRM testing in FDD cell environment |
| RRM_TDD | Used for RRM testing in TDD cell environment |
| SIG | Used for protocol testing |
| SIG_FDD | Used for protocol testing in FDD cell environment |
| SIG_TDD | Used for protocol testing in TDD cell environment |
| CEmodeA | Used for CE mode A testing using 2 CE levels |
| CEmodeB | Used for CE mode B testing using 4 CE levels |
| PRACH_2CE | Used for RRM testing using reference PRACH Configurations PRACH_2CE in TS 36.521-3 clause A.9 |
| PRACH_3CE | Used for RRM testing using reference PRACH Configurations PRACH_3CE in TS 36.521-3 clause A.9 |
| PRACH_4CE | Used for RRM testing using reference PRACH Configurations PRACH_4CE in TS 36.521-3 clause A.9 |

#### -	PRACH-ConfigSIB-v1530-DEFAULT

Table 4.6.3-7c: PRACH-ConfigSIB-v1530-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PRACH-ConfigSIB-v1530-DEFAULT ::= SEQUENCE { |  |  |  |
| edt-PRACH-ParametersListCE-r15 SEQUENCE (SIZE(1..maxCE-Level-r13)) OF SEQUENCE { | 2 entries | 1: CE level 0 2: CE level 1 | EDT AND CEmodeA |
| prach-ConfigIndex-r15[1] | 3 |  | RF |
|  | 4 |  | SIG_FDD OR RRM_FDD |
|  | 53 |  | SIG_TDD OR RRM_TDD |
| prach-FreqOffset-r15[1] | See subclause 4.6.8 | Same as prach-FrequencyOffset |  |
| prach-StartingSubframe-r15[1] | sf2 |  |  |
| mpdcch-NarrowbandsToMonitor-r15 SEQUENCE (SIZE(1..2)) OF INTEGER (1..maxAvailNarrowBands-r13)[1] | 1 entry |  |  |
| mpdcch-NarrowbandsToMonitor-r13[1] | 1 |  | RF |
|  | Value as specified in TS 36.523-3 clause 7.3.3.9 |  | SIG |
| prach-ConfigIndex-r15[2] | 3 |  | RF |
|  | 4 |  | SIG_FDD OR RRM_FDD |
|  | 53 |  | SIG_TDD OR RRM_TDD |
| prach-FreqOffset-r15[2] | See subclause 4.6.8 |  |  |
| prach-StartingSubframe-r15[2] | sf128 |  |  |
| mpdcch-NarrowbandsToMonitor-r15 SEQUENCE (SIZE(1..2)) OF INTEGER (1..maxAvailNarrowBands-r13)[2] | 1 entry |  |  |
| mpdcch-NarrowbandsToMonitor-r13[1] | 1 |  | RF |
|  | Value as specified in TS 36.523-3 clause 7.3.3.9 |  | SIG |
| } |  |  |  |
| edt-PRACH-ParametersListCE-r15 SEQUENCE (SIZE(1..maxCE-Level-r13)) OF SEQUENCE { | 4 entries | 1: CE level 0 2: CE level 1 3: CE level 2 4: CE level 3 | EDT AND CEmodeB |
| prach-ConfigIndex-r15[1] | 3 |  | RF |
|  | 4 |  | SIG_FDD OR RRM_FDD |
|  | 53 |  | SIG_TDD OR RRM_TDD |
| prach-FreqOffset-r15[1] | See subclause 4.6.8 | Same as prach-FrequencyOffset |  |
| prach-StartingSubframe-r15[1] | sf2 |  |  |
| mpdcch-NarrowbandsToMonitor-r15 SEQUENCE (SIZE(1..2)) OF INTEGER (1..maxAvailNarrowBands-r13)[1] | 1 entry |  |  |
| mpdcch-NarrowbandsToMonitor-r13[1] | 1 |  | RF |
|  | Value as specified in TS 36.523-3 clause 7.3.3.9 |  | SIG |
| prach-ConfigIndex-r15[2] | 3 |  | RF |
|  | 4 |  | SIG_FDD OR RRM_FDD |
|  | 53 |  | SIG_TDD OR RRM_TDD |
| prach-FreqOffset-r15[2] | See subclause 4.6.8 |  |  |
| prach-StartingSubframe-r15[2] | sf32 |  |  |
| mpdcch-NarrowbandsToMonitor-r15 SEQUENCE (SIZE(1..2)) OF INTEGER (1..maxAvailNarrowBands-r13)[2] | 1 entry |  |  |
| mpdcch-NarrowbandsToMonitor-r13[1] | 1 |  | RF |
|  | Value as specified in TS 36.523-3 clause 7.3.3.9 |  | SIG |
| prach-ConfigIndex-r15[3] | 3 |  | RF |
|  | 4 |  | SIG_FDD OR RRM_FDD |
|  | 53 |  | SIG_TDD OR RRM_TDD |
| prach-FreqOffset-r15[3] | See subclause 4.6.8 |  |  |
| prach-StartingSubframe-r15[3] | sf64 |  |  |
| mpdcch-NarrowbandsToMonitor-r15 SEQUENCE (SIZE(1..2)) OF INTEGER (1..maxAvailNarrowBands-r13)[3] | 1 entry |  |  |
| mpdcch-NarrowbandsToMonitor-r13[1] | 1 |  | RF |
|  | Value as specified in TS 36.523-3 clause 7.3.3.9 |  | SIG |
| prach-ConfigIndex-r15[4] | 3 |  | RF |
|  | 4 |  | SIG_FDD OR RRM_FDD |
|  | 53 |  | SIG_TDD OR RRM_TDD |
| prach-FreqOffset-r15[4] | See subclause 4.6.8 |  |  |
| prach-StartingSubframe-r15[4] | sf128 |  |  |
| mpdcch-NarrowbandsToMonitor-r15 SEQUENCE (SIZE(1..2)) OF INTEGER (1..maxAvailNarrowBands-r13)[4] | 1 entry |  |  |
| mpdcch-NarrowbandsToMonitor-r13[1] | 1 |  | RF |
|  | Value as specified in TS 36.523-3 clause 7.3.3.9 |  | SIG |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |
| RF | Used for RF, performance and RRM testing |
| RRM_FDD | Used for RRM testing in FDD cell environment |
| RRM_TDD | Used for RRM testing in TDD cell environment |
| SIG | Used for protocol testing |
| SIG_FDD | Used for protocol testing in FDD cell environment |
| SIG_TDD | Used for protocol testing in TDD cell environment |
| CEmodeA | Used for CE mode A testing using 2 CE levels |
| CEmodeB | Used for CE mode B testing using 4 CE levels |
| EDT | For Early Data Transmission testing |

#### –	PUCCH-ConfigCommon-DEFAULT

Table 4.6.3-8: PUCCH-ConfigCommon-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-ConfigCommon-DEFAULT ::= SEQUENCE { |  |  |  |
| deltaPUCCH-Shift | ds2 | Assuming typical values of the maximum delay spread |  |
| nRB-CQI | See subclause 4.6.8 | Channel-bandwidth-dependent parameter |  |
| nCS-AN | 6 |  |  |
| n1PUCCH-AN | 0 |  |  |
| } |  |  |  |

#### -	PUCCH-ConfigCommon-v1310-DEFAULT

Table 4.6.3-8A: PUCCH-ConfigCommon-v1310-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-ConfigCommon-v1310-DEFAULT ::= SEQUENCE { |  |  |  |
| n1PUCCH-AN-InfoList-r13 SEQUENCE (SIZE(1..maxCE-Level-r13)) OF INTEGER (0..2047) | 2 entries |  | CEmodeA |
| n1PUCCH-AN-InfoList-r13[1] | 2 |  |  |
| n1PUCCH-AN-InfoList-r13[2] | 2 |  |  |
| n1PUCCH-AN-InfoList-r13 SEQUENCE (SIZE(1..maxCE-Level-r13)) OF INTEGER (0..2047) | 4 entries |  | CEmodeB |
| n1PUCCH-AN-InfoList-r13[1] | 0 | INTEGER (0..2047) |  |
| n1PUCCH-AN-InfoList-r13[2] | 0 | INTEGER (0..2047) |  |
| n1PUCCH-AN-InfoList-r13[3] | 0 | INTEGER (0..2047) |  |
| n1PUCCH-AN-InfoList-r13[4] | 0 | INTEGER (0..2047) |  |
| pucch-NumRepetitionCE-Msg4-Level0-r13 | n4 |  |  |
| pucch-NumRepetitionCE-Msg4-Level1-r13 | n4 |  |  |
| pucch-NumRepetitionCE-Msg4-Level2-r13 | n16 |  | CEmodeB |
| pucch-NumRepetitionCE-Msg4-Level3-r13 | n16 |  | CEmodeB |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| CEmodeA | Used for CE mode A testing |
| CEmodeB | Used for CE mode B testing |

#### –	PUCCH-ConfigDedicated-DEFAULT

Table 4.6.3-9: PUCCH-ConfigDedicated-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-ConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| ackNackRepetition CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| tddAckNackFeedbackMode | Not present |  | FDD |
|  | multiplexing | Multiplexing is selected as default to align with RAN4’s assumptions in RF tests. | TDD |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |

#### –	PUCCH-ConfigDedicated-v1020-DEFAULT

Table 4.6.3-9A: PUCCH-ConfigDedicated-v1020-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-ConfigDedicated-v1020 ::= SEQUENCE { |  |  |  |
| pucch-Format-r10 CHOICE { |  |  | Single SCC |
| channelSelection-r10 SEQUENCE { |  |  |  |
| n1PUCCH-AN-CS-r10 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| n1PUCCH-AN-CS-List-r10 SEQUENCE{ | 1 |  | FDD |
| N1PUCCH-AN-CS-r10 | 4 |  |  |
|  | 101 |  |  |
|  | 102 |  |  |
|  | 103 |  |  |
|  | 104 |  |  |
| } |  |  |  |
| n1PUCCH-AN-CS-List-r10 SEQUENCE{ | 2 |  | 2TX AND FDD, TDD |
| N1PUCCH-AN-CS-r10 | 4 |  |  |
|  | 101 |  |  |
|  | 102 |  |  |
|  | 103 |  |  |
|  | 104 |  |  |
| N1PUCCH-AN-CS-r10 | 4 |  |  |
|  | 105 |  |  |
|  | 106 |  |  |
|  | 107 |  |  |
|  | 108 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| pucch-Format-r10 CHOICE { |  |  | 2 SCCs or more, TDD FDD CA |
| format3-r10 SEQUENCE { |  |  |  |
| n3PUCCH-AN-List-r10 SEQUENCE { | 4 |  |  |
|  | 101 |  |  |
|  | 102 |  |  |
|  | 103 |  |  |
|  | 104 |  |  |
| } |  |  |  |
| twoAntennaPortActivatedPUCCH-Format3-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| twoAntennaPortActivatedPUCCH-Format1a1b-r10 | Not present |  |  |
| simultaneousPUCCH-PUSCH-r10 | Not present |  |  |
| n1PUCCH-AN-RepP1-r10 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Single SCC | Addition or modification of single SCell |
| 2 SCCs or more | Addition or modification of SCells |
| TDD FDD CA | CA cells configured with PCell on TDD band and SCell on FDD band |

#### –	PUCCH-ConfigDedicated-v1130-DEFAULT

Table 4.6.3-9AA: PUCCH-ConfigDedicated-v1130-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-ConfigDedicated-v1130-DEFAULT ::= SEQUENCE { |  |  |  |
| n1PUCCH-AN-CS-v1130 CHOICE { |  |  |  |
| release | NULL |  |  |
| setup SEQUENCE { | 2 entries |  |  |
| n1PUCCH-AN-CS-ListP1-r11[1] | FFS |  |  |
| n1PUCCH-AN-CS-ListP1-r11[2] | FFS |  |  |
| } |  |  |  |
| } |  |  |  |
| nPUCCH-Param-r11 CHOICE { |  |  |  |
| release | NULL |  |  |
| setup SEQUENCE { |  |  |  |
| nPUCCH-Identity-r11 | FFS |  |  |
| n1PUCCH-AN-r11 | FFS |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	PUCCH-ConfigDedicated-v1250-DEFAULT

Table 4.6.3-9AB: PUCCH-ConfigDedicated-v1250-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-ConfigDedicated-v1250-DEFAULT ::= SEQUENCE { |  |  |  |
| nkaPUCCH-Param-r12 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| nkaPUCCH-AN-r12 | FFS |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	PUCCH-ConfigDedicated-r13-DEFAULT

Table 4.6.3-9AC: PUCCH-ConfigDedicated-r13-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-ConfigDedicated- r13-DEFAULT ::= SEQUENCE { |  |  |  |
| ackNackRepetition-R13 CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| tddAckNackFeedbackMode | Not present |  | FDD |
|  | multiplexing | Multiplexing is selected as default to align with RAN4’s assumptions in RF tests. | TDD |
| pucch-Format-r13 CHOICE {} | Not present |  |  |
| twoAntennaPortActivatedPUCCH-Format1a1b-r13 | Not present |  |  |
| simultaneousPUCCH-PUSCH-r13 | Not present |  |  |
| n1PUCCH-AN-RepP1-r13 | Not present |  |  |
| nPUCCH-Param-r13 CHOICE {} | Not present |  |  |
| nkaPUCCH-Param-r13 CHOICE {} | Not present |  |  |
| spatialBundlingPUCCH | FALSE |  |  |
| spatialBundlingPUSCH | FALSE |  |  |
| harq-TimingTDD | FALSE |  |  |
| codebooksizeDetermination-r13 | Not present |  |  |
| maximumPayloadCoderate-r13 | 0 |  |  |
| pucch-NumRepetitionCE-r13 CHOICE {} | Not present |  |  |
| pucch-NumRepetitionCE-r13 CHOICE { |  |  | CEmodeA, CEmodeB |
| setup CHOICE { |  |  |  |
| modeA SEQUENCE { |  |  | CEmodeA |
| pucch-NumRepetitionCE-format1-r13 | r1 | No repetitions used for protocol and performance testing to reduce the round trip time for large signaling messages. |  |
| pucch-NumRepetitionCE-format2-r13 | r1 |  |  |
| } |  |  |  |
| modeB SEQUENCE { |  |  | CEmodeB |
| pucch-NumRepetitionCE-format1-r13 | r4 |  |  |
| pucch-NumRepetitionCE-format2-r13 | r4 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| CEmodeA | Used for CE mode A testing |
| CEmodeB | Used for CE mode B testing |

#### –	PUSCH-ConfigCommon-DEFAULT

Table 4.6.3-10: PUSCH-ConfigCommon-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigCommon-DEFAULT ::= SEQUENCE { |  |  |  |
| pusch-ConfigBasic SEQUENCE { |  |  |  |
| n-SB | 1 | Typical configuration in real network |  |
| hoppingMode | interSubFrame | Typical configuration in real network |  |
| pusch-HoppingOffset | See subclause 4.6.8 | Channel-bandwidth-dependent parameter |  |
| enable64QAM | FALSE |  |  |
|  | TRUE |  | UL64QAM |
| } |  |  |  |
| ul-ReferenceSignalsPUSCH SEQUENCE { |  |  |  |
| groupHoppingEnabled | TRUE | In accordance with “the RAN5 LTE UE Feature list”. |  |
| groupAssignmentPUSCH | 0 | Typical value in real network |  |
| sequenceHoppingEnabled | FALSE | In accordance with “the RAN5 LTE UE Feature list”. |  |
| cyclicShift | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UL64QAM | Used for UL 64QAM testing |

#### –	PUSCH-ConfigCommon-v1310DEFAULT

Table 4.6.3-10A: PUSCH-ConfigCommon-v1310-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigCommon-v1310-DEFAULT ::= SEQUENCE { |  |  |  |
| pusch-maxNumRepetitionCEmodeA-r13 | Not present | Default value (r8) as defined in TS 36.213 [29, clause 8.0] apply (minimum value for CE Mode A) | CEmodeA |
| pusch-maxNumRepetitionCEmodeB-r13 | r192 | Set to minimum value for CE Mode B | CEmodeB |
| pusch-HoppingOffset-v13xy | Not present | No PUSCH frequency hopping |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| CEmodeA | Used for CE mode A testing |
| CEmodeB | Used for CE mode B testing |

#### –	PUSCH-ConfigDedicated-r13DEFAULT

Table 4.6.3-11: PUSCH-ConfigDedicated-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| betaOffset-ACK-Index | 9 |  |  |
| betaOffset-RI-Index | 6 |  |  |
| betaOffset-CQI-Index | 6 |  |  |
| } |  |  |  |

Table 4.6.3-11A: Void

#### –	PUSCH-ConfigDedicated-v1130-DEFAULT

Table 4.6.3-11AA: PUSCH-ConfigDedicated-v1130-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigDedicated-v1130-DEFAULT ::= SEQUENCE { |  |  |  |
| pusch-DMRS-r11 CHOICE { |  |  |  |
| release | NULL |  |  |
| setup SEQUENCE { |  |  |  |
| nPUSCH-Identity-r11 | FFS |  |  |
| nDMRS-CSH-Identity-r11 | FFS |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	PUSCH-ConfigDedicated-v1250-DEFAULT

Table 4.6.3-11AB: PUSCH-ConfigDedicated-v1250-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigDedicated-v1250-DEFAULT ::= SEQUENCE { |  |  |  |
| UciOnPUSCH CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| betaOffset-ACK-Index-SubframeSet2-r12 | 9 |  |  |
| betaOffset-RI-Index-SubframeSet2-r12 | 6 |  |  |
| betaOffset-CQI-Index-SubframeSet2-r12 | 6 |  |  |
| betaOffsetMC-r12 SEQUENCE { |  |  |  |
| betaOffset-ACK-Index-MC-SubframeSet2-r12 | Not present |  |  |
| betaOffset-RI-Index-MC-SubframeSet2-r12 | Not present |  |  |
| betaOffset-CQI-Index-MC-SubframeSet2-r12 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	PUSCH-ConfigDedicated--DEFAULT

Table 4.6.3-11AC: PUSCH-ConfigDedicated-r13-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigDedicated-r13-DEFAULT ::= SEQUENCE { |  |  |  |
| betaOffset-ACK-Index-r13 | 9 |  |  |
| betaOffset2-ACK-Index-r13 | Not present |  |  |
| betaOffset-RI-Index-r13 | 6 |  |  |
| betaOffset-CQI-Index-r13 | 6 |  |  |
| betaOffsetMC-r13 SEQUENCE {} | Not present |  |  |
| groupHoppingDisabled-r13 | Not present |  |  |
| dmrs-WithOCC-Activated-r13 | Not present |  |  |
| pusch-DMRS-r11 | Not present |  |  |
| uciOnPUSCH | Not present |  |  |
| pusch-HoppingConfig-r13 | Not Present | No PUSCH frequency hopping for BL UEs and UEs in CE |  |
| } |  |  |  |

#### –	PUSCH-EnhancementsConfig-r14-DEFAULT

Table 4.6.3-11AD: PUSCH-EnhancementsConfig-r14-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-EnhancementsConfig-r14-DEFAULT ::= CHOICE { |  |  |  |
| Release | NULL |  | PuschEnhancement_Release |
| Setup SEQUENCE { |  |  | PuschEnhancement _Setup |
| pusch-HoppingOffsetPUSCH-Enh-r14 | 1 |  |  |
| interval-ULHoppingPUSCH-Enh-r14 CHOICE{ |  |  |  |
| interval-FDD-PUSCH-Enh-r14 | int1 | FDD |  |
| interval-TDD-PUSCH-Enh-r14 | int1 | TDD |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| PuschEnhancement_Release | UE is not allowed to apply PUSCH enhancement |
| PuschEnhancement _Setup | UE is allowed to apply PUSCH enhancement. |

#### –	RACH-ConfigCommon-DEFAULT

Table 4.6.3-12: RACH-ConfigCommon-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| RACH-ConfigCommon-DEFAULT ::= SEQUENCE { |  |  |  |  |  |  |  |
| preambleInfo SEQUENCE { |  |  |  |  |  |  |  |
| numberOfRA-Preambles |  | n52 |  | Assuming the number of dedicated preambles is 12. |  |  |  |
| preamblesGroupAConfig SEQUENCE {} |  | Not present |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| powerRampingParameters SEQUENCE { |  |  |  |  |  |  |  |
| powerRampingStep |  | dB2 |  |  |  |  |  |
| preambleInitialReceivedTargetPower |  | dBm-104 (default) |  | Thermal noise = -113 dBm NF = 5 dB IoT = 6 dB Required SNR = -8 dB (See table 8.4.2-1 in TS 36.104 [30]) -> -110 dB (default value is acceptable) |  |  |  |
| } |  |  |  |  |  |  |  |
| ra-SupervisionInfo SEQUENCE { |  |  |  |  |  |  |  |
| preambleTransMax |  | n6 |  | Under the condition of Case 1 in RAN1 simulation assumptions, an UE with pathloss of CDF = 90% reaches the maximum transmit power in 4 successive retransmissions. 6 has been selected considering the margin of 2. |  |  |  |
| ra-ResponseWindowSize |  | sf10 |  | The maximum value is preferable. |  |  |  |
| mac-ContentionResolutionTimer |  | sf48 |  | Allows for a sufficient number of msg3 retransmissions. |  |  |  |
| } |  |  |  |  |  |  |  |
| maxHARQ-Msg3Tx |  | 4 |  | Under the condition of Target SIR = 2 dB, the probability of 2 transmissions is less than 1%. 4 has been selected considering the margin of 2. |  |  |  |
| preambleTransMax-CE-r13 |  | Not present |  |  |  |  |  |
| preambleTransMax-CE-r13 |  | n6 |  | Same as preambleTransMax |  | CEmodeA or CEmodeB |  |
| RACH-CE-LevelInfoList-r13 |  | Not present |  |  |  |  |  |
| RACH-CE-LevelInfoList-r13 SEQUENCE (SIZE (1..maxCE-Level-r13)) OF RACH-CE-LevelInfo-r13 |  | 2 entries |  | first two 2 entries [1] and [2] of list to be used |  | CEmodeA |  |
| RACH-CE-LevelInfoList-r13 SEQUENCE (SIZE (1..maxCE-Level-r13)) OF RACH-CE-LevelInfo-r13 |  | 4 entries |  |  |  | CEmodeB |  |
| RACH-CE-LevelInfo-r13[1] SEQUENCE { |  |  |  |  |  |  |  |
| preambleMappingInfo-r13 SEQUENCE { |  |  |  |  |  |  |  |
| firstPreamble-r13 |  | 0 |  |  |  |  |  |
| lastPreamble-r13 |  | 14 |  |  |  |  |  |
| lastPreamble-r13 |  | 13 |  |  |  | EDT |  |
| } |  |  |  |  |  |  |  |
| ra-ResponseWindowSize-r13 |  | sf20 |  |  |  |  |  |
| mac-ContentionResolutionTimer-r13 |  | sf80 |  | Allows for a sufficient number of msg3 retransmissions. Longer time for increasing CE levels. |  |  |  |
| rar-HoppingConfig-r13 |  | off |  |  |  |  |  |
| edt-Parameters-r15 SEQUENCE { |  |  |  |  |  | EDT |  |
| edt-LastPreamble-r15 |  | 14 |  |  |  |  |  |
| edt-SmallTBS-Enabled-r15 |  | False |  |  |  |  |  |
| edt-TBS-r15 |  | b408 |  |  |  |  |  |
| mac-ContentionResolutionTimer-r15 |  | Not present |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| RACH-CE-LevelInfo-r13[2] SEQUENCE { |  |  |  |  |  |  |  |
| preambleMappingInfo-r13 SEQUENCE { |  |  |  |  |  |  |  |
| firstPreamble-r13 |  | 15 |  |  |  |  |  |
| lastPreamble-r13 |  | 29 |  |  |  |  |  |
| lastPreamble-r13 |  | 28 |  |  |  | EDT |  |
| } |  |  |  |  |  |  |  |
| ra-ResponseWindowSize-r13 |  | sf80 |  |  |  |  |  |
| mac-ContentionResolutionTimer-r13 |  | sf120 |  |  |  |  |  |
| rar-HoppingConfig-r13 |  | off |  |  |  |  |  |
| edt-Parameters-r15 SEQUENCE { |  |  |  |  |  | EDT |  |
| edt-LastPreamble-r15 |  | 29 |  |  |  |  |  |
| edt-SmallTBS-Enabled-r15 |  | False |  |  |  |  |  |
| edt-TBS-r15 |  | b408 |  |  |  |  |  |
| mac-ContentionResolutionTimer-r15 |  | Not present |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| RACH-CE-LevelInfo-r13[3] SEQUENCE { |  |  |  |  |  |  |  |
| PreambleMappingInfo-r13[3] SEQUENCE { |  |  |  |  |  |  |  |
| firstPreamble-r13 |  | 30 |  |  |  |  |  |
| lastPreamble-r13 |  | 44 |  |  |  |  |  |
| lastPreamble-r13 |  | 43 |  |  |  | EDT |  |
| } |  |  |  |  |  |  |  |
| ra-ResponseWindowSize-r13 |  | sf180 |  |  |  |  |  |
| mac-ContentionResolutionTimer-r13 |  | sf200 |  |  |  |  |  |
| rar-HoppingConfig-r13 |  | off |  |  |  |  |  |
| edt-Parameters-r15 SEQUENCE { |  |  |  |  |  | EDT |  |
| edt-LastPreamble-r15 |  | 44 |  |  |  |  |  |
| edt-SmallTBS-Enabled-r15 |  | False |  |  |  |  |  |
| edt-TBS-r15 |  | b408 |  |  |  |  |  |
| mac-ContentionResolutionTimer-r15 |  | Not present |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| RACH-CE-LevelInfo-r13[4] SEQUENCE { |  |  |  |  |  |  |  |
| preambleMappingInfo-r13 SEQUENCE { |  |  |  |  |  |  |  |
| firstPreamble-r13 |  | 45 |  |  |  |  |  |
| lastPreamble-r13 |  | 60 |  |  |  |  |  |
| lastPreamble-r13 |  | 59 |  |  |  | EDT |  |
| } |  |  |  |  |  |  |  |
| ra-ResponseWindowSize-r13 |  | sf320 |  |  |  |  |  |
| mac-ContentionResolutionTimer-r13 |  | sf480 |  |  |  |  |  |
| rar-HoppingConfig-r13 |  | off |  |  |  |  |  |
| edt-Parameters-r15 SEQUENCE { |  |  |  |  |  | EDT |  |
| edt-LastPreamble-r15 |  | 60 |  |  |  |  |  |
| edt-SmallTBS-Enabled-r15 |  | False |  |  |  |  |  |
| edt-TBS-r15 |  | b408 |  |  |  |  |  |
| mac-ContentionResolutionTimer-r15 |  | Not present |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| edt-SmallTBS-Subset-r15 |  | Not present |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| CEmodeA | Used for CE mode A testing |
| CEmodeB | Used for CE mode B testing |
| EDT | For Early Data Transmission testing |

#### –	Rach-ConfigDedicated-DEFAULT

Table 4.6.3-12A: Rach-ConfigDedicated-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Rach-ConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| ra-PreambleIndex | 52 |  |  |
| ra-PRACH-MaskIndex | 0 |  |  |
| } |  |  |  |

#### –	RadioResourceConfigCommon-DEFAULT

Table 4.6.3-13: RadioResourceConfigCommon-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| RadioResourceConfigCommon-DEFAULT ::= SEQUENCE { |  |  |  |  |  |  |  |
| rach-ConfigCommon | RACH-ConfigCommon-DEFAULT |  |  |  |  |  |
| prach-Config | PRACH-Config-DEFAULT |  |  |  |  |  |
| pdsch-ConfigCommon | Not present |  |  |  |  |  |
|  | PDSCH-ConfigCommon-DEFAULT |  |  |  | FullConfig, HO-to-EUTRA |  |
| pusch-ConfigCommon | PUSCH-ConfigCommon-DEFAULT |  |  |  |  |  |
| phich-Config | Not present |  |  |  |  |  |
|  | PHICH-Config-DEFAULT |  |  |  | FullConfig, HO-to-EUTRA |  |
| pucch-ConfigCommon | Not present |  |  |  | SAME-BW |  |
|  | PUCCH-ConfigCommon-DEFAULT |  |  |  | DIFF-BW, FullConfig, HO-to-EUTRA |  |
| soundingRSUL-ConfigCommon | SoundingRS-UL-ConfigCommon-DEFAULT |  |  |  |  |  |
| uplinkPowerControlCommon | Not present |  |  |  |  |  |
|  | UplinkPowerControlCommon-DEFAULT |  |  |  | FullConfig, HO-to-EUTRA |  |
| antennaInfoCommon | Not present |  |  |  |  |  |
| antennaInfoCommon SEQUENCE { |  |  |  |  | 2TX |  |
| antennaPortsCount | an2 |  |  |  |  |  |
| } |  |  |  |  |  |  |
| antennaInfoCommon SEQUENCE { |  |  |  |  | 4TX |  |
| antennaPortsCount | an4 |  |  |  |  |  |
| } |  |  |  |  |  |  |
| antennaInfoCommon SEQUENCE { |  |  |  |  | FullConfig, HO-to-EUTRA |  |
| antennaPortsCount | an1 |  |  |  |  |  |
| } |  |  |  |  |  |  |
| p-Max | Not present |  |  |  |  |  |
| tdd-Config | Not present |  |  |  | FDD or LAA SCell |  |
|  | TDD-Config-DEFAULT |  |  |  | TDD |  |
| ul-CyclicPrefixLength | len1 |  |  |  |  |  |
| uplinkPowerControlCommon-v1020 | UplinkPowerControlCommon-v1020-DEFAULT |  |  |  | CA |  |
|  | Not present |  |  |  |  |  |
| tdd-Config-v1130 | Not present |  |  |  |  |  |
| pusch-ConfigCommon-v1270 | Not present |  |  |  |  |  |
| prach-Config-v1310 | Not present |  |  |  |  |  |
|  | PRACH-Config-v1310-DEFAULT |  |  |  | CEmodeA, CEmodeB |  |
| freqHoppingParameters-r13 | Not present |  |  |  |  |  |
|  | FreqHoppingParameters-r13-DEFAULT |  |  |  | (CEmodeA OR CEmodeB) AND SIG |  |
| pdsch-ConfigCommon-v1310 | Not present |  |  |  |  |  |
|  | PDSCH-ConfigCommon-v1310-DEFAULT |  |  |  | CEmodeA, CEmodeB |  |
| pucch-ConfigCommon-v1310 | Not present |  |  |  |  |  |
|  | PUCCH-ConfigCommon-v1310-DEFAULT |  |  |  | CEmodeA, CEmodeB |  |
| pusch-ConfigCommon-v1310 | Not present |  |  |  |  |  |
|  | PUSCH-ConfigCommon-v1310-DEFAULT |  |  |  | CEmodeA, CEmodeB |  |
| uplinkPowerControlCommon-v1310 | Not present |  |  |  |  |  |
| highSpeedConfig-r14 | Not present |  |  |  |  |  |
|  | HighSpeedConfig-r14-DEFAULT |  |  |  | HighSpeedEnhance |  |
| prach-Config-v1430 | Not present |  |  |  |  |  |
|  | PRACH-Config-v1430-DEFAULT |  |  |  | HighSpeedEnhance |  |
| uplinkPowerControlCommon-v1530 | Not present |  |  |  |  |  |
|  | uplinkPowerControlCommon-v1530-DEFAULT |  |  |  | Short-TTI |  |
| } |  |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |
| 2TX | Used for cells with two antenna ports |
| 4TX | Used for cells with four antenna ports |
| SAME-BW | Source and target cell are configured with same bandwidth |
| DIFF-BW | Source and target cell are configured with different bandwidth |
| UL CA | When UL Carrier Aggregation is used. |
| FullConfig | Handover with full configuration option (Rel-9 or later) |
| HO-to-EUTRA | Inter-RAT handover to E-UTRA |
| CA | For Carrier Aggregation Test cases |
| CEmodeA | Used for CE mode A testing |
| CEmodeB | Used for CE mode B testing |
| LAA SCell | For LAA SCell environment |
| SIG | For Signalling test cases |
| HighSpeedEnhance | High speed test environment |
| Short-TTI | When short TTI is used |

#### –	RadioResourceConfigCommonSCell-r10-DEFAULT

Table 4.6.3-13A: RadioResourceConfigCommonSCell-r10-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSCell-r10-DEFAULT ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| dl-Bandwidth-r10 | Same downlink bandwidth as used for target SCell |  |  |
| antennaInfoCommon-r10 SEQUENCE { |  |  |  |
| antennaPortsCount | an1 |  | 1TX |
| antennaPortsCount | an2 |  | 2TX |
| antennaPortsCount | an4 |  | 4TX |
| } |  |  |  |
| mbsfn-SubframeConfigList-r10 | Not present |  |  |
| phich-Config-r10 | PHICH-Config-DEFAULT |  |  |
| pdsch-ConfigCommon-r10 | PDSCH-ConfigCommon-DEFAULT |  |  |
| tdd-Config-r10 | Not present |  | FDD |
|  | TDD-Config-DEFAULT |  | TDD |
| } |  |  |  |
| ul-Configuration-r10 | Not present |  |  |
| ul-Configuration-r10 SEQUENCE { |  |  | UL CA |
| ul-FreqInfo-r10 SEQUENCE { |  |  |  |
| ul-CarrierFreq-r10 | Not present |  |  |
| ul-Bandwidth-r10 | Not present |  |  |
| additionalSpectrumEmissionSCell-r10 | 1 (CA_NS_01) |  |  |
| } |  |  |  |
| p-Max-r10 | Not present |  |  |
| uplinkPowerControlCommonSCell-r10 | UplinkPowerControlCommonSCell-r10-DEFAULT |  |  |
| soundingRS-UL-ConfigCommon-r10 | SoundingRS-UL-ConfigCommon-DEFAULT |  |  |
| ul-CyclicPrefixLength-r10 | len1 |  |  |
| prach-ConfigSCell-r10 | Not present |  | FDD |
|  | FFS |  | TDD |
| pusch-ConfigCommon-r10 | PUSCH-ConfigCommon-DEFAULT |  |  |
| } |  |  |  |
| pusch-ConfigCommon-v1270 | Not present |  |  |
| pusch-ConfigCommon-v1270 SEQUENCE { |  |  | UL64QAM |
| enable64QAM-v1270 | true |  |  |
| } |  |  |  |
| pucch-ConfigCommon-r13 | Not present |  |  |
|  | PUCCH-ConfigCommon-DEFAULT |  | PUCCH SCell |
| uplinkPowerControlCommonSCell-v1310 | Not present |  |  |
|  | UplinkPowerControlCommonSCell-v1310 |  | PUCCH SCell |
| ul-Configuration-r14 SEQUENCE { |  |  | SRS-Switching |
| ul-FreqInfo-r14 SEQUENCE { |  |  |  |
| ul-CarrierFreq-r14 | Not present |  |  |
| ul-Bandwidth-r14 | Not present |  |  |
| additionalSpectrumEmissionSCell-r14 | 1 (CA_NS_01) |  |  |
| } |  |  |  |
| p-Max-r14 | Not present |  |  |
| soundingRS-UL-ConfigCommon-r14 | SoundingRS-UL-ConfigCommon-DEFAULT |  |  |
| ul-CyclicPrefixLength-r14 | len1 |  |  |
| prach-ConfigSCell-r14 | Not Present |  |  |
| uplinkPowerControlCommonPUSCH-LessCell-v14xy SEQUENCE { |  |  |  |
| p0-Nominal-PeriodicSRS-r14 | -85 | Same as default PUSCH p0-Norminal |  |
| p0-Nominal-AperiodicSRS-r14 | -85 | Same as default PUSCH p0-Norminal |  |
| alpha-SRS-r14 | al08 |  |  |
| } |  |  |  |
| harq-ReferenceConfig-r14 | Not Present |  |  |
| soundingRS-FlexibleTiming-r14 | Not Present |  |  |
| } |  |  |  |
| highSpeedConfigSCell-r14 | Not Present |  |  |
|  | HighSpeedConfigSCell-r14-DEFAULT |  | HighSpeedEnhance |
| prach-Config-v1430 | Not present |  |  |
|  | PRACH-Config-v1430-DEFAULT |  | HighSpeedEnhance |
| } |  |  |  |

| Condition | Explanation |  |
| --- | --- | --- |
| FDD | FDD cell environment |  |
| TDD | TDD cell environment |  |
| UL CA | When UL Carrier Aggregation is used. |  |
| 1TX | Used for SCell with one antenna port |  |
| 2TX | Used for SCell with two antenna ports |  |
| 4TX | Used for SCell with four antenna ports |  |
| SRS-Switching | SRS transmission is switched to a PUSCH-less TDD SCell. |  |
| HighSpeedEnhance | High speed test environment |  |
| PUCCH-SCell |  | When this SCell is configured as PUCCH SCell |  |
| UL64QAM |  | Used for UL 64QAM testing for SCell |  |

#### –	RadioResourceConfigCommonSIB-DEFAULT

Table 4.6.3-14: RadioResourceConfigCommonSIB-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSIB-DEFAULT ::= SEQUENCE { |  |  |  |
| rach-ConfigCommon | RACH-ConfigCommon-DEFAULT |  |  |
| bcch-Config | BCCH-Config-DEFAULT |  |  |
| pcch-Config | PCCH-Config-DEFAULT |  |  |
| prach-Config | PRACH-ConfigSIB-DEFAULT |  |  |
| pdsch-ConfigCommon | PDSCH-ConfigCommon-DEFAULT |  |  |
| pusch-ConfigCommon | PUSCH-ConfigCommon-DEFAULT |  |  |
| pucch-ConfigCommon | PUCCH-ConfigCommon-DEFAULT |  |  |
| soundingRS-UL-ConfigCommon | SoundingRS-UL-ConfigCommon-DEFAULT |  |  |
| uplinkPowerControlCommon | UplinkPowerControlCommon-DEFAULT |  |  |
| ul-CyclicPrefixLength | len1 |  |  |
| uplinkPowerControlCommon-v1020 | Not present |  |  |
|  | UplinkPowerControlCommon-v1020-DEFAULT |  | CA |
| rach-ConfigCommon-v1250 | Not present |  |  |
| pusch-ConfigCommon-v1270 | Not present |  |  |
| pusch-ConfigCommon-v1270 SEQUENCE { |  |  | UL64QAM |
| enable64QAM-v1270 | true |  |  |
| } |  |  |  |
| bcch-Config-v1310 | Not present |  |  |
| pcch-Config-v1310 | Not present |  |  |
|  | PCCH-Config-v1310-DEFAULT |  | CEmodeA, CEmodeB |
| freqHoppingParameters-r13 | Not Present |  |  |
|  | FreqHoppingParameters-r13-DEFAULT |  | CEmodeA, CEmodeB |
| pdsch-ConfigCommon-v1310 | Not present |  |  |
|  | PDSCH-ConfigCommon-v1310-DEFAULT |  | CEmodeA, CEmodeB |
| pusch-ConfigCommon-v1310 | Not present |  |  |
|  | PUSCH-ConfigCommon-v1310-DEFAULT |  | CEmodeA, CEmodeB |
| prach-ConfigCommon-v1310 | Not present |  |  |
|  | PRACH-ConfigSIB-v1310-DEFAULT |  | CEmodeA, CEmodeB |
| pucch-ConfigCommon-v1310 | Not present |  |  |
|  | PUCCH-ConfigCommon-v1310-DEFAULT |  | CEmodeA, CEmodeB |
| highSpeedConfig-r14 | Not present |  |  |
|  | HighSpeedConfig-r14-DEFAULT |  | HighSpeedEnhance |
| prach-Config-v1430 | Not present |  |  |
|  | PRACH-Config-v1430-DEFAULT |  | HighSpeedEnhance |
| prach-Config-v1530 | Not present |  |  |
|  | PRACH-ConfigSIB-v1530-DEFAULT |  | EDT |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| CA | For Carrier Aggregation test cases |
| CEmodeA | Used for CE mode A testing |
| CEmodeB | Used for CE mode B testing |
| HighSpeedEnhance | High speed test environment |
| UL64QAM | Used for UL 64QAM testing |
| EDT | For Early Data Transmission testing |

#### –	RadioResourceConfigDedicated-SRB1

Table 4.6.3-15: RadioResourceConfigDedicated-SRB1

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-SRB1 ::= SEQUENCE { |  |  |  |  |
| srb-ToAddModList SEQUENCE (SIZE (1..2)) OF SEQUENCE {} |  | 1 entry, with value SRB-ToAddMod-DEFAULT using condition SRB1 |  |  |
| drb-ToAddModList |  | Not present |  |  |
| drb-ToReleaseList |  | Not present |  |  |
| mac-MainConfig CHOICE { |  |  |  |  |
| explicitValue |  | MAC-MainConfig-SRB | See subclause 4.8.2 |  |
| } |  |  |  |  |
| sps-Config |  | Not present |  |  |
| physicalConfigDedicated |  | PhysicalConfigDedicated-DEFAULT using condition SRB1 | For signalling test cases see subclause 6.6B.1.1.1. Otherwise, see subclause 4.8.2 |  |
| } |  |  |  |  |

#### –	RadioResourceConfigDedicated-SRB2-DRB(n,m)

Table 4.6.3-16: RadioResourceConfigDedicated-SRB2-DRB(n,m)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-SRB2-DRB(n, m) ::= SEQUENCE { |  |  | n is the number of AM RLC DRBs (1..N) m is the number of UM RLC DRBs (0..M) |  |
| srb-ToAddModList SEQUENCE (SIZE (1..2)) OF SEQUENCE { |  | 1 entry |  |  |
| srb-ToAddMod[1] |  | SRB-ToAddMod-DEFAULT using condition SRB2 | See subclause 4.8.2 |  |
| } |  |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF SEQUENCE { |  | n+m entries |  |  |
| drb-ToAddMod[k, k=1..n] |  | DRB-ToAddMod-DEFAULT (k) using condition AM | n AM RLC DRBs See subclause 4.8.2 |  |
| drb-ToAddMod[k, k=n+1..n+m] |  | DRB-ToAddMod-DEFAULT (k) using condition UM | m UM RLC DRBs See subclause 4.8.2 | m>0 |
| } |  |  |  |  |
| drb-ToReleaseList |  | Not present |  |  |
| mac-MainConfig CHOICE { |  |  |  |  |
| explicitValue |  | MAC-MainConfig-RBC using condition DRX_L (note) | See subclause 4.8.2 | m=0 |
|  |  | MAC-MainConfig-RBC using condition DRX_S (note) | See subclause 4.8.2 | m>0 |
| } |  |  |  |  |
| sps-Config |  | Not present |  |  |
| physicalConfigDedicated |  | PhysicalConfigDedicated-DEFAULT using condition RBC | For signalling test cases see subclause 6.6B.1.1.1. Otherwise, see subclause 4.8.2 |  |
| } |  |  |  |  |
| NOTE: In cases where no RLC-UM bearer is configured, large DRX Cycle length is used. In cases where at least one RLC-UM bearer is configured, small DRX Cycle length is used. |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| m=0 | m is equal to zero |
| m>0 | m is greater than zero |

#### –	RadioResourceConfigDedicated-DRB(n,m)

Table 4.6.3-17: RadioResourceConfigDedicated-DRB(n,m)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-DRB(n, m) ::= SEQUENCE { |  |  | n is the number of AM RLC DRBs (0..N) m is the number of UM RLC DRBs (0..M) |  |
| srb-ToAddModList |  | Not present |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF SEQUENCE { |  | n+m entries | BID is the total number of established DRBs in the UE, before applying the contents of this IE |  |
| drb-ToAddMod[k, k=BID+1..BID+n] |  | DRB-ToAddMod-DEFAULT (k) using condition AM | n AM RLC DRBs See subclause 4.8.2 | n>0 |
| drb-ToAddMod[k, k= BID+1+n.. BID+n+m] |  | DRB-ToAddMod-DEFAULT (k) using condition UM | m UM RLC DRBs See subclause 4.8.2 | m>0 |
| } |  |  |  |  |
| drb-ToReleaseList |  | Not present |  |  |
| mac-MainConfig |  | Not present (note) |  | m=0 |
| mac-MainConfig CHOICE { |  |  |  | m>0 |
| explicitValue |  | MAC-MainConfig-RBC using condition DRX_S (note) | See subclause 4.8.2 |  |
| } |  |  |  |  |
| sps-Config |  | Not present |  |  |
| physicalConfigDedicated |  | Not present |  |  |
| } |  |  |  |  |
| NOTE: In cases where no RLC-UM bearer is added, existing DRX configuration is used without modification. In cases where at least one RLC-UM bearer is added, small DRX Cycle length is used. |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| n>0 | n is greater than zero |
| m=0 | m is equal to zero |
| m>0 | m is greater than zero |

#### –	RadioResourceConfigDedicated-SRB4

Table 4.6.3-17A: RadioResourceConfigDedicated-SRB4

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-SRB4 ::= SEQUENCE { |  |  |  |  |
| srb-ToAddModExtList-r15 SEQUENCE (SIZE (1)) OF SEQUENCE { |  | 1 entry |  |  |
| srb-ToAddMod |  | SRB4-ToAddMod-DEFAULT | See subclause 4.8.2 |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	RadioResourceConfigDedicated-HO-TO-EUTRA(n,m)

Table 4.6.3-18: RadioResourceConfigDedicated-HO-TO-EUTRA(n,m)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-HO-TO-EUTRA(n, m) ::= SEQUENCE { |  |  | n is the number of AM RLC DRBs (1..N) m is the number of UM RLC DRBs (0..M) |  |
| srb-ToAddModList SEQUENCE (SIZE (1..2)) OF SEQUENCE { |  | 2 entries |  |  |
| srb-ToAddMod[1] |  | SRB-ToAddMod-DEFAULT using condition SRB1 | See subclause 4.8.2 |  |
| srb-ToAddMod[2] |  | SRB-ToAddMod-DEFAULT using condition SRB2 | See subclause 4.8.2 |  |
| } |  |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF SEQUENCE { |  | n+m entries |  |  |
| drb-ToAddMod[k, k=1..n] |  | DRB-ToAddMod-DEFAULT (k) using condition AM | n AM RLC DRBs See subclause 4.8.2 |  |
| drb-ToAddMod[k, k=n+1..n+m] |  | DRB-ToAddMod-DEFAULT (k) using condition UM | m UM RLC DRBs See subclause 4.8.2 | m>0 |
| } |  |  |  |  |
| drb-ToReleaseList |  | Not present |  |  |
| mac-MainConfig CHOICE { |  |  |  |  |
| explicitValue |  | MAC-MainConfig-RBC using condition DRX_L (note) | See subclause 4.8.2 | m=0 |
|  |  | MAC-MainConfig-RBC using condition DRX_S (note) | See subclause 4.8.2 | m>0 |
| } |  |  |  |  |
| sps-Config |  | Not present |  |  |
| physicalConfigDedicated |  | PhysicalConfigDedicated-DEFAULT using condition RBC | For signalling test cases see subclause 6.6B.1.1.1. Otherwise, see subclause 4.8.2 |  |
| } |  |  |  |  |
| NOTE: In cases where no RLC-UM bearer is configured, large DRX Cycle length is used. In cases where at least one RLC-UM bearer is configured, small DRX Cycle length is used. |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| m=0 | m is equal to zero |
| m>0 | m is greater than zero |

#### -	RadioResourceConfigDedicated-AM-DRB-ADD(bid)

Table 4.6.3-18A: RadioResourceConfigDedicated-AM-DRB-ADD(bid)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-AM-DRB-ADD(bid) ::= SEQUENCE { |  |  | bid is the bearer identity |  |
| srb-ToAddModList |  | Not present |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF SEQUENCE { |  | one entry |  |  |
| drb-ToAddMod[1] |  | DRB-ToAddMod-DEFAULT (bid) using condition AM | See subclause 4.8.2 |  |
| } |  |  |  |  |
| drb-ToReleaseList |  | Not present |  |  |
| mac-MainConfig |  | Not present |  |  |
| sps-Config |  | Not present |  |  |
| physicalConfigDedicated |  | Not present |  |  |
| } |  |  |  |  |

#### -	RadioResourceConfigDedicated-UM-DRB-ADD(bid)

Table 4.6.3-18B: RadioResourceConfigDedicated-UM-DRB-ADD(bid)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-UM-DRB-ADD(bid) ::= SEQUENCE { |  |  | bid is the bearer identity |  |
| srb-ToAddModList |  | Not present |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF SEQUENCE { |  | one entry |  |  |
| drb-ToAddMod[1] |  | DRB-ToAddMod-DEFAULT (bid) using condition UM | See subclause 4.8.2 |  |
| } |  |  |  |  |
| drb-ToReleaseList |  | Not present |  |  |
| mac-MainConfig CHOICE { |  |  |  |  |
| explicitValue |  | MAC-MainConfig-RBC using condition DRX_S (note) | See subclause 4.8.2 |  |
| } |  |  |  |  |
| sps-Config |  | Not present |  |  |
| physicalConfigDedicated |  | Not present |  |  |
| } |  |  |  |  |
| NOTE: Since one RLC-UM bearer is added, small DRX Cycle length is used. |  |  |  |  |

#### -	RadioResourceConfigDedicated- DRB-REL(bid)

Table 4.6.3-18C: RadioResourceConfigDedicated-DRB-REL(bid)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-DRB-REL(bid) ::= SEQUENCE { |  |  | bid is the bearer identity |  |
| srb-ToAddModList |  | Not present |  |  |
| drb-ToAddModList |  | Not present |  |  |
| drb-ToReleaseList SEQUENCE (SIZE (1..maxDRB)) OF |  | one entry |  |  |
| drb-Identity[1] |  | bid |  |  |
| mac-MainConfig |  | Not present |  |  |
| sps-Config |  | Not present |  |  |
| physicalConfigDedicated |  | Not present |  |  |
| } |  |  |  |  |

#### –	RadioResourceConfigDedicated-HO

Table 4.6.3-19: RadioResourceConfigDedicated-HO

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-HO ::= SEQUENCE { |  |  |  |
| srb-ToAddModList | Not present |  |  |
| drb-ToAddModList | Not present |  |  |
| drb-ToReleaseList | Not present |  |  |
| mac-MainConfig | Not present |  |  |
| sps-Config | Not present |  |  |
| physicalConfigDedicated | PhysicalConfigDedicated-DEFAULT using condition RBC-HO | For signalling test cases see subclause 6.6B.1.1.1. Otherwise, see subclause 4.8.2 |  |
| } |  |  |  |

#### –	RadioResourceConfigDedicatedSCell-r10-DEFAULT

Table 4.6.3-19AA: RadioResourceConfigDedicatedSCell-r10-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicatedSCell-r10 ::= SEQUENCE { |  |  |  |
| physicalConfigDedicatedSCell-r10 | PhysicalConfigDedicatedSCell-r10-DEFAULT |  |  |
| } |  |  |  |

#### –	RadioResourceConfigDedicated-SCell_AddMod

Table 4.6.3-19AAA: RadioResourceConfigDedicated-SCell_AddMod

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-SCell_AddMod ::= SEQUENCE { |  |  |  |
| srb-ToAddModList | Not present |  |  |
| drb-ToAddModList | Not present |  |  |
| drb-ToReleaseList | Not present |  |  |
| mac-MainConfig | MAC-MainConfig-RBC using condition SCell_AddMod | See subclause 4.8.2 |  |
| sps-Config | Not present |  |  |
| physicalConfigDedicated | PhysicalConfigDedicated-DEFAULT using condition SCell_AddMod | For signalling test cases see subclause 6.6B.1.1.1. Otherwise, see subclause 4.8.2 |  |
| } |  |  |  |

Table 4.6.3-19AAAA: Void

#### –	RadioResourceConfigDedicated-V2X

Table 4.6.3-19AAAAA: RadioResourceConfigDedicated-V2X-SPS

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment |  | Condition |
| RadioResourceConfigDedicated-V2X-SPS::= SEQUENCE { |  |  |  |  |
| sps-Config-v1430 SEQUENCE { |  |  |  |  |
| ul-SPS-V-RNTI-r14 |  | Not Present |  |  |  |
| sl-SPS-V-RNTI-r14 |  | '1034'H |  |  |  |
| sps-ConfigUL-ToAddModList-r14 |  | Not Present |  |  |  |
| sps-ConfigUL-ToReleaseList-r14 | Not Present |  |  |  |
| sps-ConfigSL-ToAddModList-r14 SEQUENCE (SIZE (1..maxConfigSPS-r14)) OF SEQUENCE { |  |  |  | SL_ADD |
| sps-ConfigIndex-r14 | 1 | INTEGER (1..maxConfigSPS-r14) |  |  |
| semiPersistSchedIntervalSL-r14 | sf20 | ENUMERATED { sf20 } |  |  |
| } |  |  |  |  |
| sps-ConfigSL-ToReleaseList-r14 SEQUENCE (SIZE (1..maxConfigSPS-r14)) OF SEQUENCE { |  |  |  | SL_RELEASE |
| SPS-ConfigIndex-r14 | 1 |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| SL_ADD | Add SPS configuration for V2X communication over Sidelink |
| SL_RELEASE | Release SPS configuration for V2X communication over Sidelink |

#### -	RadioResourceConfigDedicated-EN-DC-Split

Table 4.6.3-19AAAAAB: RadioResourceConfigDedicated-EN-DC-Split

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-EN-DC-Split ::= SEQUENCE { |  |  |  |  |
| srb-ToAddModList SEQUENCE (SIZE (1..2)) OF SEQUENCE { |  | 1 entry |  |  |
| srb-ToAddMod[1] |  | SRB-ToAddMod-DEFAULT using condition SRB2 | See subclause 4.8.2 |  |
| } |  |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF SEQUENCE { |  | 2 entries |  |  |
| drb-ToAddMod[1] |  | DRB-ToAddMod-DEFAULT (8) using condition AM | See subclause 4.8.2 |  |
| drb-ToAddMod[2] |  | DRB-ToAddMod-EN-DC-Split | See subclause 4.8.2 |  |
| } |  |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF SEQUENCE { |  | 3 entries |  | MULTI_PDN |
| drb-ToAddMod[1] |  | DRB-ToAddMod-DEFAULT (1) using condition AM | See subclause 4.8.2 |  |
| drb-ToAddMod[2] |  | DRB-ToAddMod-DEFAULT (8) using condition AM | See subclause 4.8.2 |  |
| drb-ToAddMod[3] |  | DRB-ToAddMod-EN-DC-Split | See subclause 4.8.2 |  |
| } |  |  |  |  |
| drb-ToReleaseList |  | Not present |  |  |
| mac-MainConfig CHOICE { |  |  |  |  |
| explicitValue |  | MAC-MainConfig-RBC using condition DRX_L | See subclause 4.8.2 |  |
| } |  |  |  |  |
| sps-Config |  | Not present |  |  |
| physicalConfigDedicated |  | PhysicalConfigDedicated-DEFAULT using condition RBC | For signalling test cases see subclause 6.6B.1.1.1. Otherwise, see subclause 4.8.2 |  |
| } |  |  |  |  |

#### -	RadioResourceConfigDedicated-EN-DC_Split_DRB_Rel(bid)

Table 4.6.3-19AAAAAC: RadioResourceConfigDedicated-DRB-REL(bid)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-EN-DC_Split_DRB_Rel(bid) ::= SEQUENCE { |  |  | bid is the bearer identity of EN-DC Split DRB |  |
| srb-ToAddModList |  | Not present |  |  |
| drb-ToAddModList |  | Not present |  |  |
| drb-ToReleaseList SEQUENCE (SIZE (1..maxDRB)) OF |  | one entry |  |  |
| drb-Identity[1] |  | bid |  |  |
| mac-MainConfig |  | Not present |  |  |
| sps-Config |  | Not present |  |  |
| physicalConfigDedicated |  | Not present |  |  |
| } |  |  |  |  |

#### –	RadioResourceConfigDedicated-EN-DC-SRB2-DRB

Table 4.6.3-19AAAAAD: RadioResourceConfigDedicated-EN-DC-SRB2-DRB

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-SRB2-DRB ::= SEQUENCE { |  |  |  |  |
| srb-ToAddModList SEQUENCE (SIZE (1..2)) OF SEQUENCE { |  | 1 entry |  |  |
| srb-ToAddMod[1] |  | SRB-ToAddMod-DEFAULT using condition SRB2 | See subclause 4.8.2 |  |
| } |  |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF SEQUENCE { |  | 1 entry |  |  |
| drb-ToAddMod[1] |  | DRB-ToAddMod-DEFAULT (8) using condition AM | See subclause 4.8.2 |  |
| } |  |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF SEQUENCE { |  | 2 entries |  | MULTI_PDN |
| drb-ToAddMod[1] |  | DRB-ToAddMod-DEFAULT (1) using condition AM | See subclause 4.8.2 |  |
| drb-ToAddMod[2] |  | DRB-ToAddMod-DEFAULT (8) using condition AM | See subclause 4.8.2 |  |
| } |  |  |  |  |
| drb-ToReleaseList |  | Not present |  |  |
| mac-MainConfig CHOICE { |  |  |  |  |
| explicitValue |  | MAC-MainConfig-RBC using condition DRX_L SCG_Add_EN-DC | See subclause 4.8.2 |  |
| } |  |  |  |  |
| sps-Config |  | Not present |  |  |
| physicalConfigDedicated |  | PhysicalConfigDedicated-DEFAULT using condition RBC | For signalling test cases see subclause 6.6B.1.1.1. Otherwise, see subclause 4.8.2 |  |
| } |  |  |  |  |

#### –	RLC-Config-DRB-AM-RECONFIG

Table 4.6.3-19A: RLC-Config-DRB-AM-RECONFIG

| Derivation Path: 36.508 table 4.8.2.1.3.2-1: RLC-Config-DRB-AM |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RLC-Config-DRB-AM-RECONFIG ::= CHOICE { |  |  |  |  |
| am SEQUENCE { |  |  |  |  |
| ul-AM-RLC SEQUENCE { |  |  |  |  |
| t-PollRetransmit |  | ms85 |  |  |
| pollPDU |  | p64 |  |  |
| pollByte |  | kB250 |  |  |
| maxRetxThreshold |  | t8 |  |  |
| } |  |  |  |  |
| dl-AM-RLC SEQUENCE { |  |  |  |  |
| t-Reordering |  | ms85 |  |  |
| t-StatusProhibit |  | ms50 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	RLC-Config-DRB-UM-RECONFIG

Table 4.6.3-19B: RLC-Config-DRB-UM-RECONFIG

| Derivation Path: 36.508 table 4.8.2.1.3.1-1: RLC-Config-DRB-UM |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RLC-Config-DRB-UM-RECONFIG ::= CHOICE { |  |  |  |  |
| um-Bi-Directional SEQUENCE { |  |  |  |  |
| ul-UM-RLC SEQUENCE { |  |  |  |  |
| sn-FieldLength |  | Size10 |  |  |
| } |  |  |  |  |
| dl-UM-RLC SEQUENCE { |  |  |  |  |
| sn-FieldLength |  | Size10 |  |  |
| t-Reordering |  | ms55 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### –	RLC-Config-SRB-AM-RECONFIG

Table 4.6.3-19C: RLC-Config-SRB-AM-RECONFIG

| Derivation Path: 36.331 clause 6.3.2, 9.2.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLC-Config-SRB-AM-RECONFIG ::= CHOICE { |  |  |  |
| am SEQUENCE { |  |  |  |
| ul-AM-RLC SEQUENCE { |  |  |  |
| t-PollRetransmit | ms50 |  |  |
| pollPDU | pInfinity | Default |  |
| pollByte | kBinfinity | Default |  |
| maxRetxThreshold | t6 |  |  |
| } |  |  |  |
| dl-AM-RLC SEQUENCE { |  |  |  |
| t-Reordering | ms40 |  |  |
| t –StatusProhibit | ms0 | Default |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	SCellToAddMod-r10-DEFAULT

Table 4.6.3-19D: SCellToAddMod-r10-DEFAULT

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SCellToAddMod-r10 ::= SEQUENCE { |  |  |  |
| sCellIndex-r10 | Set according to specific message content |  |  |
| cellIdentification-r10 SEQUENCE { |  |  |  |
| physCellId-r10 | Set according to specific message content |  |  |
| dl-CarrierFreq-r10 | Set according to specific message content |  |  |
| dl-CarrierFreq-r10 | 65535 | maxEARFCN | Band > 64 |
| } |  |  |  |
| radioResourceConfigCommonSCell-r10 | RadioResourceConfigCommonSCell-r10-DEFAULT |  |  |
| radioResourceConfigDedicatedSCell-r10 | RadioResourceConfigDedicatedSCell-r10-DEFAULT |  |  |
| dl-CarrierFreq-v1090 | Set according to specific message content |  | Band > 64 |
| antennaInfoDedicatedSCell-v10i0 ::= SEQUENCE { |  |  | 4Rx_SCC |
| maxLayersMIMO-r10 | fourLayers |  |  |
| } |  |  |  |
| srs-SwitchFromServCellIndex-r14 | 0 |  | SRS-Switching |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |
| 4Rx_SCC | For 4Rx testcases with more than 2 Layers on SCC |
| SRS-Switching | SRS transmission is switched to a PUSCH-less TDD SCell. |

#### –	SCellToRelease-r10-DEFAULT

Table 4.6.3-19E: SCellToRelease-r10-DEFAULT

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| sCellToReleaseList-r10 SEQUENCE (SIZE (1..maxSCell-r10) OF SEQUENCE { |  |  |  |
| sCellIndex-r10[1] | 1 |  |  |
| } |  |  |  |

#### –	SCG-Configuration-r12-DEFAULT

Table 4.6.3-19F: SCG-Configuration-r12-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Information Element | Value/remark |  | Comment | Condition |  |
| scg-Configuration-r12 CHOICE { |  |  |  |  |
| release | NULL |  |  | PSCell_Release |
| setup SEQUENCE { |  |  |  | PSCell_Mod, PSCell_Add_Split_DRB, PSCell_Add_SCG_DRB |
| scg-ConfigPartMCG-r12 SEQUENCE { |  |  |  |  |
| scg-Counter-r12 | 0 | INTEGER (0.. 65535) |  |  |
| powerCoordinationInfo-r12 SEQUENCE { |  |  |  |  |
| p-MeNB-r12 | 6 (30%) | INTEGER (1..16) mapped to 0..15 in Table 5.1.4.2-1 in TS 36.213 for power allocation for DC (0% - 100%) |  |  |
| p-SeNB-r12 | 6 (30%) | INTEGER (1..16) mapped to 0..15 in Table 5.1.4.2-1 in TS 36.213 for power allocation for DC (0% - 100%) |  |  |
| powerControlMode-r12 | 1 | DC power control mode 1 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| scg-ConfigPartSCG-r12 SEQUENCE { |  |  |  | PSCell_Add_Split_DRB, PSCell_Add_SCG_DRB |
| radioResourceConfigDedicatedSCG-r12 SEQUENCE { |  |  |  |  |
| drb-ToAddModListSCG-r12 SEQUENCE (SIZE (1..maxDRB)) OF SEQUENCE { | 1 entry |  |  |  |
| DRB-ToAddModSCG-r12[1] SEQUENCE { |  |  |  |  |
| drb-Identity-r12 | Same DRB ID as used for the MCG |  |  | PSCell_Add_Split_DRB |
| drb-Identity-r12 | BID+1 | BID is the total number of established DRBs in the UE, before applying the contents of this IE |  | PSCell_Add_SCG_DRB |
| drb-Type-r12 CHOICE { |  |  |  |  |
| split-r12 | NULL |  |  | PSCell_Add_Split_DRB |
| scg-r12 SEQUENCE { |  |  |  | PSCell_Add_SCG_DRB |
| eps-BearerIdentity-r12 | BID+5 | drb-Identity-r12+4 |  |  |
| pdcp-Config-r12 | PDCP-Config-DRB-AM |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| rlc-ConfigSCG-r12 | RLC-Config-DRB-AM |  |  |  |
| rlc-Config-v1250 | Not present |  |  |  |
| logicalChannelIdentitySCG-r12 | 2 | drb-Identity-r12+2 |  |  |
| logicalChannelConfigSCG-r12 | LogicalChannelConfig-DRB using condition LO |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| mac-MainConfigSCG-r12 | Not present |  |  |  |
| rlf-TimersAndConstantsSCG-r12 | Not present |  |  |  |
| } |  |  |  |  |
| sCellToReleaseListSCG-r12 | Not Present |  |  |  |
| pSCellToAddMod-r12 SEQUENCE { |  |  |  |  |
| sCellIndex-r12 | Set according to specific message content |  |  |  |
| cellIdentification-r12 SEQUENCE { |  |  |  |  |
| physCellId-r12 | Set according to specific message content |  |  |  |
| dl-CarrierFreq-r12 | Set according to specific message content |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| radioResourceConfigCommonPSCell-r12 SEQUENCE { |  |  |  |  |
| basicFields-r12 | RadioResourceConfigCommonSCell-r10-DEFAULT |  |  |  |
| pucch-ConfigCommon-r12 | PUCCH-ConfigCommon-DEFAULT |  |  |  |
| rach-ConfigCommon-r12 | RACH-ConfigCommon-DEFAULT |  |  |  |
| uplinkPowerControlCommonPSCell-r12 SEQUENCE { |  |  |  |  |
| deltaF-PUCCH-Format3-r12 | deltaF0 | Same value as in UplinkPowerControlCommon-v1020-DEFAULT used for the PCell |  | RM coding |
|  | deltaF4 |  |  | Dual RM coding |
| deltaF-PUCCH-Format1bCS-r12 | deltaF1 | Same value as in UplinkPowerControlCommon-v1020-DEFAULT used for the PCell |  |  |
| p0-NominalPUCCH-r12 | -117 (-117 dBm) | Same value as in UplinkPowerControlCommon--DEFAULT used for the PCell |  |  |
| deltaFList-PUCCH-r12 SEQUENCE { |  |  |  |  |
| deltaF-PUCCH-Format1 | deltaF0 | Same value as in UplinkPowerControlCommon-DEFAULT used for the PCell |  |  |
| deltaF-PUCCH-Format1b | deltaF3 | Same value as in UplinkPowerControlCommon-DEFAULT used for the PCell |  |  |
| deltaF-PUCCH-Format2 | deltaF0 | Same value as in UplinkPowerControlCommon-DEFAULT used for the PCell |  |  |
| deltaF-PUCCH-Format2a | deltaF0 | Same value as in UplinkPowerControlCommon-DEFAULT used for the PCell |  |  |
| deltaF-PUCCH-Format2b | deltaF0 | Same value as in UplinkPowerControlCommon-DEFAULT used for the PCell |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| radioResourceConfigDedicatedPSCell-r12 SEQUENCE { |  |  |  |  |
| physicalConfigDedicatedPSCell-r12 | PhysicalConfigDedicated-DEFAULT | For signalling test cases see subclause 6.6B.1.1.1. Otherwise, see subclause 4.8.2 |  |  |
| sps-Config-r12 | Not present |  |  |  |
| naics-Info-r12 | Not present |  |  |  |
| } |  |  |  |  |
| antennaInfoDedicatedPSCell-v1280 | Not present |  |  |  |
| } |  |  |  |  |
| sCellToAddModListSCG-r12 | Not Present |  |  |  |
| mobilityControlInfoSCG-r12 |  |  |  |  |
| t307-r12 | ms2000 | ENUMERATED {ms50, ms100, ms150, ms200, ms500,ms1000, ms2000} |  |  |
| ue-IdentitySCG-r12 | Any allowed value |  |  |  |
| rach-ConfigDedicated-r12 | Rach-ConfigDedicated-DEFAULT |  |  |  |
| cipheringAlgorithmSCG-r12 | Set according to PIXIT parameter for default ciphering algorithm |  |  | For SIG |
|  | eea0 |  |  | For RF |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| PSCell_Mod | Modification of PSCell(s) |
| PSCell_Release | Release of PSCell |
| PSCell_Add_Split_DRB | Add PSCell(s) and setup of Split DRB |
| PSCell_Add_SCG_DRB | Add PSCell(s) and setup of SCG DRB |
| RM coding | Used for Reed-Muller coding |
| Dual RM coding | Used for Dual Reed-Muller coding |
| For SIG | Used for signalling test cases |
| For RF | Used for RF/RRM test cases |

#### –	SchedulingRequest-Config-DEFAULT

Table 4.6.3-20: SchedulingRequest-Config-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequest-Config-DEFAULT ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sr-PUCCH-ResourceIndex | See subclause 4.6.8 | Channel-bandwidth-dependent parameter |  |
| sr-ConfigIndex | 30 |  | FDD |
|  | 32 |  | FDD and (CEmodeA or CEmodeB) and 3 MHz BW |
|  | 27 |  | TDD |
| dsr-TransMax | n4 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |
| CEmodeA | Used for CE mode A testing |
| CEmodeB | Used for CE mode B testing |
| 3 MHz BW | Used for cells with 3 MHz bandwidth |

Table 4.6.3-20A: Void

#### –	SchedulingRequestConfigSCell-r13-DEFAULT

Table 4.6.3-20B: SchedulingRequestConfigSCell-r13-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequestConfigSCell-r13 ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sr-PUCCH-ResourceIndex-r13 | See subclause 4.6.8 | Channel-bandwidth-dependent parameter |  |
| sr-PUCCH-ResourceIndexP1-r13 | Not present |  |  |
| sr-ConfigIndex-r13 | 30 |  | FDD |
|  | 27 |  | TDD |
| dsr-TransMax-13 | n4 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |

Table 4.6.3-20B: Void

#### –	SL-CommResourcePoolV2X-r14-DEFAULT

Table 4.6.3-20C: SL-CommResourcePoolV2X-r14-DEFAULT

| Derivation Path: 36.331 clause 6.3.8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-CommResourcePoolV2X-r14-DEFAULT ::= SEQUENCE { |  |  |  |
| sl-OffsetIndicator-r14 | Not present |  |  |
| sl-Subframe-r14 CHOICE { |  |  |  |
| bs20-r14 | 11110000000000000000 |  | BITMAP_1 |
|  | 00001111000000000000 |  | BITMAP_2 |
|  | 00000000111100000000 |  | BITMAP_3 |
|  | 00000000000011110000 |  | BITMAP_4 |
|  | 00000000000000001111 |  | BITMAP_5 |
|  | 11111111111111111111 |  | BITMAP_6 |
| } |  |  |  |
| adjacencyPSCCH-PSSCH-r14 | true | BOOLEAN |  |
| sizeSubchannel-r14 | n5 |  | BW10 |
|  | n10 |  | BW20 |
| numSubchannel-r14 | n10 |  |  |
| startRB-Subchannel-r14 | 0 |  |  |
| startRB-PSCCH-Pool-r14 | Not present |  |  |
| rxParametersNCell-r14 | Not present |  |  |
| dataTxParameters-r14 {} | Not present |  |  |
| dataTxParameters-r14 SEQUENCE { |  |  | COND_TX |
| alpha-r12 | al0 |  |  |
| p0-r12 | 31 |  |  |
| } |  |  |  |
| zoneID-r14 | Not present | INTEGER (0..7) |  |
| threshS-RSSI-CBR-r14 | Not present | INTEGER (0..45) |  |
| poolReportId-r14 | Not present |  |  |
| cbr-pssch-TxConfigList-r14 | Not present |  |  |
| resourceSelectionConfigP2X-r14 | Not present |  |  |
| syncAllowed-r14 | Not present |  |  |
| restrictResourceReservationPeriod-r14 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| BW10 | 10 MHz channel bandwidth for V2X communication |
| BW20 | 20 MHz channel bandwidth for V2X communication |

#### –	SL-CommTxPoolSensingConfig-r14-DEFAULT

Table 4.6.3-20D: SL-CommTxPoolSensingConfig-r14-DEFAULT

| Derivation Path: 36.331 clause 6.3.8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-CommTxPoolSensingConfig-r14-DEFAULT ::= SEQUENCE { |  |  |  |
| pssch-TxConfigList-r14 | SL-PSSCH-TxConfig-r14-DEFAULT |  |  |
| thresPSSCH-RSRP-List-r14 SEQUENCE (SIZE (64)) OF SL-ThresPSSCH-RSRP-r14 { |  |  |  |
| SL-ThresPSSCH-RSRP-r14[n] | 1 | For n=1,2,…,64, where n denotes the index for the threshold used for sensing based UE autonomous resource selection |  |
| } |  |  |  |
| restrictResourceReservationPeriod-r14 SEQUENCE (SIZE (1..maxReservationPeriod-r14)) OF SL-RestrictResourceReservationPeriod-r14 { |  |  |  |
| SL-RestrictResourceReservationPeriod-r14[1] | v0dot2 |  |  |
| SL-RestrictResourceReservationPeriod-r14[2] | v0dot5 |  |  |
| SL-RestrictResourceReservationPeriod-r14[3] | v1 |  |  |
| SL-RestrictResourceReservationPeriod-r14[4] | v2 |  |  |
| SL-RestrictResourceReservationPeriod-r14[5] | v3 |  |  |
| SL-RestrictResourceReservationPeriod-r14[6] | v4 |  |  |
| SL-RestrictResourceReservationPeriod-r14[7] | v5 |  |  |
| SL-RestrictResourceReservationPeriod-r14[8] | v6 |  |  |
| SL-RestrictResourceReservationPeriod-r14[9] | v7 |  |  |
| SL-RestrictResourceReservationPeriod-r14[10] | v8 |  |  |
| SL-RestrictResourceReservationPeriod-r14[11] | v9 |  |  |
| SL-RestrictResourceReservationPeriod-r14[12] | v10 |  |  |
| } |  |  |  |
| probResourceKeep-r14 | v0 |  |  |
| p2x-SensingConfig-r14 | Not present |  |  |
| sl-ReselectAfter-r14 | Not present |  |  |
| } |  |  |  |

#### –	SL-InterFreqInfoV2X-r14-DEFAULT

Table 4.6.3-20E: SL-InterFreqInfoV2X-r14-DEFAULT

| Derivation Path: 36.331 clause 6.3.8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-InterFreqInfoV2X-r14-DEFAULT ::= SEQUENCE { |  |  |  |
| plmn-IdentityList-r14 | Not present |  |  |
| v2x-CommCarrierFreq-r14 | set according to the specific test configuration |  |  |
| sl-MaxTxPower-r14 | Not present | INTEGER (-30..33) |  |
| sl-Bandwidth-r14 | n50 | ENUMERATED {n6, n15, n25, n50, n75, n100} | BW10 |
|  | n100 |  | BW20 |
| v2x-SchedulingPool-r14 | SL-CommResourcePoolV2X-r14-DEFAULT using conditions BITMAP_4 and COND_TX |  | SCHEDULED |
|  | Not present |  | SIB |
| v2x-UE-ConfigList-r14 | SL-V2X-InterFreqUE-Config-r14-DEFAULT using condition SIB |  | SIB |
|  | SL-V2X-InterFreqUE-Config-r14-DEFAULT using condition RRC |  | RRC |
| additionalSpectrumEmissionV2X-r14 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SIB | Inter-frequency V2X configuration for SIB21 |
| RRC | Inter-frequency V2X configuration for RRCConnectionReconfiguration |
| BW10 | 10 MHz channel bandwidth for V2X communication |
| BW20 | 20 MHz channel bandwidth for V2X communication |
| SCHEDULED | The resources the UE can use are scheduled by the NW. |

#### –	SlotOrSubslotPDSCH-Config-r15-DEFAULT

Table 4.6.3-20EA: SlotOrSubslotPDSCH-Config-r15-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SlotOrSubslotPDSCH-Config-r15-DEFAULT ::= SEQUENCE { |  |  |  |
| altCQI-TableSTTI-r15 | Not present |  |  |
|  | allSubframes |  | DL 256QAM |
| altCQI-Table1024QAM-STTI-r15 | Not present |  |  |
| resourceAllocation-r15 | resourceAllocationType0 |  |  |
| tbsIndexAlt-STTI-r15 | Not present |  |  |
| tbsIndexAlt2-STTI-r15 | Not present |  |  |
| tbsIndexAlt3-STTI-r15 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| DL 256QAM | When DL 256QAM is used |

#### –	SlotOrSubslotPUSCH-Config-r15-DEFAULT

Table 4.6.3-20EB: SlotOrSubslotPUSCH-Config-r15-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SlotOrSubslotPUSCH-Config-r15-DEFAULT ::= SEQUENCE { |  |  |  |
| betaOffsetSlot-ACK-Index-r15 | 9 |  |  |
| betaOffset2Slot-ACK-Index-r15 | Not present |  |  |
| betaOffsetSubslot-ACK-Index-r15 SEQUENCE (SIZE(1..2)) OF | 1 entry |  |  |
|  | 9 |  |  |
| betaOffset2Subslot-ACK-Index-r15 | Not present |  |  |
| betaOffsetSlot-RI-Index-r15 | 6 |  |  |
| betaOffsetSubslot-RI-Index-r15 SEQUENCE (SIZE(1..2)) OF | 1 entry |  |  |
|  | 6 |  |  |
| betaOffsetSlot-CQI-Index-r15 | 6 |  |  |
| betaOffsetSubslot-CQI-Index-r15 SEQUENCE (SIZE(1..2)) OF | 1 entry |  |  |
|  | 6 |  |  |
| enable256QAM-SlotOrSubslot-r15 | Not present |  |  |
| resourceAllocationOffset-r15 | Not present |  |  |
| ul-DMRS-IFDMA-SlotOrSubslot-r15 | FALSE |  |  |
| } |  |  |  |

#### –	SPDCCH-Config-r15-DEFAULT

Table 4.6.3-20EC: SPDCCH-Config-r15-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SPDCCH-Config-r15-DEFAULT ::= SEQUENCE { |  |  |  |
| spdcch-L1-ReuseIndication-r15 | n1 |  |  |
| spdcch-SetConfig-r15 SEQUENCE (SIZE(1..4)) OF SEQUENCE { | 1 entry |  |  |
| spdcch-SetConfigId-r15 | 0 |  |  |
| spdcch-SetReferenceSig-r15 | crs |  |  |
| transmissionType-r15 | localised |  |  |
| spdcch-NoOfSymbols-r15 | 1 |  |  |
| dmrs-ScramblingSequenceInt-r15 | Not present |  |  |
| dci7-CandidatesPerAL-PDCCH-r15 SEQUENCE (SIZE(1..4)) OF | 4 entries |  |  |
|  | 1 | AL=1 |  |
|  | 2 | AL=2 |  |
|  | 0 | AL=4 |  |
|  | 0 | AL=8 |  |
| dci7-CandidateSetsPerAL-SPDCCH-r15 SEQUENCE (SIZE(1..2)) OF | 2 entries |  |  |
| DCI7-CandidatesPerAL-SPDCCH-r15 SEQUENCE (SIZE(1..4)) OF | 4 entries |  |  |
|  | 1 | AL=1 |  |
|  | 0 | AL=2 |  |
|  | 1 | AL=4 |  |
|  | 0 | AL=8 |  |
| DCI7-CandidatesPerAL-SPDCCH-r15 SEQUENCE (SIZE(1..4)) OF | 4 entries |  |  |
|  | 1 | AL=1 |  |
|  | 1 | AL=2 |  |
|  | 0 | AL=4 |  |
|  | 0 | AL=8 |  |
| resourceBlockAssignment-r15 SEQUENCE { |  |  |  |
| numberRB-InFreq-domain-r15 | 8 |  |  |
| resourceBlockAssignment-r15 | 11100100101110110011001101100 |  |  |
| } |  |  |  |
| subslotApplicability-r15 | 11001 |  |  |
| al-StartingPointSPDCCH-r15 SEQUENCE (SIZE(1..4)) OF | 4 entries |  |  |
|  | 2 | AL=1 |  |
|  | 2 | AL=2 |  |
|  | 0 | AL=4 |  |
|  | 0 | AL=8 |  |
| subframeType-r15 | nonmbsfn |  |  |
| rateMatchingMode-r15 | m2 |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	SPUCCH-Config-r15-DEFAULT

Table 4.6.3-20ED: SPUCCH-Config-r15-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SPUCCH-Config-r15-DEFAULT ::= SEQUENCE { |  |  |  |
| spucch-Set-r15 SEQUENCE (SIZE(1..4)) OF SEQUENCE { | 1 entry |  |  |
| n1SubslotSPUCCH-AN-List-r15 SEQUENCE (SIZE(1..4)) OF | 4 entries |  |  |
|  | 101 | INTEGER (0..1319) |  |
|  | 102 |  |  |
|  | 103 |  |  |
|  | 104 |  |  |
| n1SlotSPUCCH-FH-AN-List-r15 | 101 |  |  |
| n1SlotSPUCCH-NoFH-AN-List-r15 | 101 |  |  |
| n3SPUCCH-AN-List-r15 | 101 |  |  |
| n4SPUCCHSlot-Resource-r15 | Not present |  |  |
| n4SPUCCHSubslot-Resource-r15 | Not present |  |  |
| n4maxCoderateSlotPUCCH-r15 | Not present |  |  |
| n4maxCoderateSubslotPUCCH-r15 | Not present |  |  |
| n4maxCoderateMultiResourceSlotPUCCH-r15 | Not present |  |  |
| n4maxCoderateMultiResourceSubslotPUCCH-r15 | Not present |  |  |
| } |  |  |  |
| twoAntennaPortActivatedSPUCCH-Format1a1b | Not present |  |  |
| twoAntennaPortActivatedSPUCCH-Format1a3 | Not present |  |  |
| } |  |  |  |

#### –	SL-PSSCH-TxConfig-r14-DEFAULT

Table 4.6.3-20F: SL-PSSCH-TxConfig-r14-DEFAULT

| Derivation Path: 36.331 clause 6.3.8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-PSSCH-TxConfig-r14-DEFAULT ::= SEQUENCE { |  |  |  |
| typeTxSync-r14 | enb |  |  |
| thresUE-Speed-r14 | kmph200 |  |  |
| parametersAboveThres-r14 SEQUENCE { |  |  |  |
| minMCS-PSSCH-r14 | 0 |  |  |
| maxMCS-PSSCH-r14 | 15 |  |  |
| minSubChannel-NumberPSSCH-r14 | 1 |  |  |
| maxSubchannel-NumberPSSCH-r14 | 50 |  | BW10 |
|  | 100 |  | BW20 |
| allowedRetxNumberPSSCH-r14 | both | ENUMERATED {n0, n1, both, spare1} | RF |
|  | n0 |  | SIG |
| maxTxPower-r14 | Not present |  |  |
| } |  |  |  |
| parametersBelowThres-r14 SEQUENCE { |  |  |  |
| minMCS-PSSCH-r14 | 4 |  |  |
| maxMCS-PSSCH-r14 | 25 |  |  |
| minSubChannel-NumberPSSCH-r14 | 1 |  |  |
| maxSubchannel-NumberPSSCH-r14 | 50 |  | BW10 |
|  | 100 |  | BW20 |
| allowedRetxNumberPSSCH-r14 | n1 | ENUMERATED {n0, n1, both, spare1} | RF |
|  | n0 |  | SIG |
| maxTxPower-r14 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| BW10 | 10 MHz channel bandwidth for V2X communication |
| BW20 | 20 MHz channel bandwidth for V2X communication |
| RF | For RF, performance and RRM testing |
| SIG | For protocol testing |

#### –	SL-TxPoolToAddMod-r14-DEFAULT

Table 4.6.3-20G: SL-TxPoolToAddMod-r14-DEFAULT

| Derivation Path: 36.331 clause 6.3.8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-TxPoolToAddMod-r14-DEFAULT ::= SEQUENCE { |  |  |  |
| poolIdentity-r14 | 1 |  |  |
| pool-r14 | SL-CommResourcePoolV2X-r14-DEFAULT using conditions BITMAP_5 and COND_TX |  |  |
| } |  |  |  |

#### –	SL-TxPoolToReleaseListV2X-r14-DEFAULT

Table 4.6.3-20H: SL-TxPoolToReleaseListV2X-r14-DEFAULT

| Derivation Path: 36.331 clause 6.3.8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-TxPoolToReleaseListV2X-r14-DEFAULT ::= SEQUENCE (SIZE (1..maxSL-V2X-TxPool-r14)) OF SL-V2X-TxPoolIdentity-r14 { |  |  |  |
| SL-V2X-TxPoolIdentity-r14[1] | 1 |  |  |
| } |  |  |  |

#### –	SL-V2X-ConfigDedicated-r14-DEFAULT

Table 4.6.3-20HA: SL-V2X-ConfigDedicated-r14-DEFAULT

| Derivation Path: 36.331 clause 6.3.8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-V2X-ConfigDedicated-r14-DEFAULT ::= SEQUENCE { |  |  |  |
| commTxResources-r14 CHOICE { | Not present |  |  |
| commTxResources-r14 CHOICE { |  |  | SCHEDULED |
| release | NULL |  | RELEASE |
| setup CHOICE { |  |  | SETUP |
| scheduled-r14 SEQUENCE { |  |  |  |
| sl-V-RNTI-r14 | '1111'H | BIT STRING (SIZE (16)) |  |
| mac-MainConfig-r14 SEQUENCE { |  |  |  |
| periodic-BSR-TimerSL | sf20 |  |  |
| retx-BSR-TimerSL | sf20 |  |  |
| } |  |  |  |
| v2x-SchedulingPool-r14 | Not present |  |  |
| mcs-r14 | Not present | INTEGER (0..31) |  |
| logicalChGroupInfoList-r14 SEQUENCE (SIZE (1.. maxLCG-r13)) OF SL-PriorityList-r13 |  | 4 entries |  |
| SL-PriorityList-r13[1] ::= SEQUENCE (SIZE (1..maxSL-Prio-r13)) OF SL-Priority-r13 { |  |  |  |
| SL-Priority-r13[1] | 1 |  |  |
| SL-Priority-r13[2] | 2 |  |  |
| } |  |  |  |
| SL-PriorityList-r13[2] ::= SEQUENCE (SIZE (1..maxSL-Prio-r13)) OF SL-Priority-r13 { |  |  |  |
| SL-Priority-r13[1] | 3 |  |  |
| SL-Priority-r13[2] | 4 |  |  |
| } |  |  |  |
| SL-PriorityList-r13[3] ::= SEQUENCE (SIZE (1..maxSL-Prio-r13)) OF SL-Priority-r13 { |  |  |  |
| SL-Priority-r13[1] | 5 |  |  |
| SL-Priority-r13[2] | 6 |  |  |
| } |  |  |  |
| SL-PriorityList-r13[4] ::= SEQUENCE (SIZE (1..maxSL-Prio-r13)) OF SL-Priority-r13 { |  |  |  |
| SL-Priority-r13[1] | 7 |  |  |
| SL-Priority-r13[2] | 8 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ue-Selected-r14 | Not present |  |  |  |
| } |  |  |  |
| } |  |  |  |  |
| v2x-InterFreqInfoList-r14 SEQUENCE (SIZE (0..maxFreqV2X-1-r14)) OF SL-InterFreqInfoV2X-r14 { |  |  |  |
| SL-InterFreqInfoV2X-r14[1] | SL-InterFreqInfoV2X-r14-DEFAULT using condition RRC |  | UE-SELECTED |
| SL-InterFreqInfoV2X-r14[1] | SL-InterFreqInfoV2X-r14-DEFAULT using condition RRC and SCHEDULED |  | SCHEDULED |
| } |  |  |  |
| thresSL-TxPrioritization-r14 | Not present |  |  |
| typeTxSync-r14 | enb |  |  |
| cbr-DedicatedTxConfigList-r14 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SCHEDULED | The resources the UE can use are scheduled by the NW. |
| UE-SELECTED | The resources the UE can use are to be selected by the UE from set of resources provided by the NW. |
| RELEASE | Release Communication Resources for V2X Sidelink communication |
| SETUP | Setup Communication Resources for V2X Sidelink communication |

#### –	SL-V2X-InterFreqUE-Config-r14-DEFAULT

Table 4.6.3-20I: SL-V2X-InterFreqUE-Config-r14-DEFAULT

| Derivation Path: 36.331 clause 6.3.8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-V2X-InterFreqUE-Config-r14-DEFAULT ::= SEQUENCE { |  |  |  |
| physCellIdList-r14 | Not present |  |  |
| typeTxSync-r14 | enb |  |  |
| v2x-SyncConfig-r14 | Not present |  |  |
| v2x-CommRxPool-r14 SEQUENCE (SIZE (1..maxSL-V2X-RxPool-r14)) OF SL-CommResourcePoolV2X-r14 { |  |  |  |
| SL-CommResourcePoolV2X-r14[1] | SL-CommResourcePoolV2X-r14-DEFAULT using conditions BITMAP_1 |  |  |
| SL-CommResourcePoolV2X-r14[2] | SL-CommResourcePoolV2X-r14-DEFAULT using conditions BITMAP_2 |  |  |
| SL-CommResourcePoolV2X-r14[3] | SL-CommResourcePoolV2X-r14-DEFAULT using conditions BITMAP_3 |  |  |
| SL-CommResourcePoolV2X-r14[4] | SL-CommResourcePoolV2X-r14-DEFAULT using conditions BITMAP_4 |  |  |
| SL-CommResourcePoolV2X-r14[5] | SL-CommResourcePoolV2X-r14-DEFAULT using conditions BITMAP_5 |  |  |
| } |  |  |  |
| v2x-CommTxPoolNormal-r14 SEQUENCE (SIZE (1..maxSL-V2X-TxPool-r14)) OF SL-CommResourcePoolV2X-r14 { |  |  |  |
| SL-CommResourcePoolV2X-r14[1] | SL-CommResourcePoolV2X-r14-DEFAULT using conditions BITMAP_1 and COND_TX |  | SIB |
|  | SL-CommResourcePoolV2X-r14-DEFAULT using conditions BITMAP_5 and COND_TX |  | RRC |
| } |  |  |  |
| p2x-CommTxPoolNormal-r14 | Not present |  |  |
| v2x-CommTxPoolExceptional-r14 | SL-CommResourcePoolV2X-r14-DEFAULT using conditions BITMAP_3 and COND_TX |  |  |
| v2x-ResourceSelectionConfig-r14 | SL-CommTxPoolSensingConfig-r14-DEFAULT |  |  |
| zoneConfig-r14 | Not present |  |  |
| offsetDFN-r14 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SIB | UE configuration for Sidelink communication mode 4 for SIB21 |
| RRC | UE configuration for Sidelink communication mode 4 for RRCConnectionReconfiguration |

#### –	SL-V2X-PreconfigCommPool-r14-DEFAULT

Table 4.6.3-20J: SL-V2X-PreconfigCommPool-r14-DEFAULT

| Derivation Path: 36.331 clause 9.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-V2X-PreconfigCommPool-r14-DEFAULT ::= SEQUENCE { |  |  |  |
| sl-OffsetIndicator-r14 | Not present |  |  |
| sl-Subframe-r14 CHOICE { |  |  |  |
| bs20-r14 | 11110000000000000000 |  | BITMAP_1 |
|  | 00001111000000000000 |  | BITMAP_2 |
|  | 00000000111100000000 |  | BITMAP_3 |
|  | 00000000000011110000 |  | BITMAP_4 |
|  | 00000000000000001111 |  | BITMAP_5 |
|  | 11111111111111111111 |  | BITMAP_6 |
| } |  |  |  |
| adjacencyPSCCH-PSSCH-r14 | true | BOOLEAN |  |
| sizeSubchannel-r14 | n5 |  | BW10 |
|  | n10 |  | BW20 |
| numSubchannel-r14 | n10 |  |  |
| startRB-Subchannel-r14 | 0 |  |  |
| startRB-PSCCH-Pool-r14 | Not present |  |  |
| dataTxParameters-r14 | 31 | p0-r12 |  |
| zoneID-r14 | Not present | INTEGER (0..7) |  |
| threshS-RSSI-CBR-r14 | Not present | INTEGER (0..45) |  |
| cbr-pssch-TxConfigList-r14 | Not present |  |  |
| resourceSelectionConfigP2X-r14 | Not present |  |  |
| syncAllowed-r14 | Not present |  |  |
| restrictResourceReservationPeriod-r14 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| BW10 | 10 MHz channel bandwidth cell environment |
| BW20 | 20 MHz channel bandwidth cell environment |

#### –	SL-V2X-PreconfigFreqInfo-r14-DEFAULT

Table 4.6.3-20K: SL-V2X-PreconfigFreqInfo-r14-DEFAULT

| Derivation Path: 36.331 clause 9.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-V2X-PreconfigFreqInfo-r14-DEFAULT ::= SEQUENCE { |  |  |  |
| v2x-CommPreconfigGeneral-r14 SEQUENCE { |  | SL-PreconfigGeneral-r12 |  |
| rohc-Profiles-r12 SEQUENCE { |  | -- PDCP configuration |  |
| profile0x0001-r12 | false |  |  |
| profile0x0002-r12 | false |  |  |
| profile0x0004-r12 | false |  |  |
| profile0x0006-r12 | false |  |  |
| profile0x0101-r12 | false |  |  |
| profile0x0102-r12 | false |  |  |
| profile0x0104-r12 | false |  |  |
| } |  |  |  |
| carrierFreq-r12 | f1 as per table 6.2.3.5-1 | ARFCN-ValueEUTRA-r9 |  |
| maxTxPower-r12 | 26 |  |  |
| additionalSpectrumEmission-r12 | 1(NS_01) | A-MPR doesn’t apply by default. See TS 36.101 table 6.2.4-1 |  |
| sl-bandwidth-r12 | Set according to specific message content |  |  |
| tdd-ConfigSL-r12 SEQUENCE { |  | TDD-ConfigSL-r12 |  |
| subframeAssignment | none |  |  |
| reserved-r12 | ‘0000000000000000000’B |  |  |
| additionalSpectrumEmission-v1440 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| v2x-CommPreconfigSync-r14 | Not present |  |  |
| v2x-CommRxPoolList-r14 SEQUENCE (SIZE (1..maxSL-V2X-RxPoolPreconf-r14)) OF SL-V2X-PreconfigCommPool-r14 { | 1 entry |  |  |
| SL-V2X-PreconfigCommPool-r14[1] | SL-V2X-PreconfigCommPool-r14-DEFAULT using condition BITMAP_6 |  |  |
| } |  |  |  |
| v2x-CommTxPoolList-r14 SEQUENCE (SIZE (1..maxSL-V2X-TxPoolPreconf-r14)) OF SL-V2X-PreconfigCommPool-r14 { |  |  |  |
| SL-V2X-PreconfigCommPool-r14[1] | SL-V2X-PreconfigCommPool-r14-DEFAULT using conditions BITMAP_6 and COND_TX |  |  |
| } |  |  |  |
| p2x-CommTxPoolList-r14 | Not present |  |  |
| v2x-ResourceSelectionConfig-r14 | SL-CommTxPoolSensingConfig-r14-DEFAULT |  |  |
| zoneConfig-r14 | Not present |  |  |
| syncPriority-r14 | gnss |  |  |
| thresSL-TxPrioritization-r14 | Not present |  |  |
| offsetDFN-r14 | Not present |  |  |
| } |  |  |  |

#### –	SoundingRS-UL-ConfigCommon-DEFAULT

Table 4.6.3-21: SoundingRS-UL-ConfigCommon-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SoundingRS-UL-ConfigCommon-DEFAULT ::= SEQUENCE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| -srs-BandwidthConfig | See subclause 4.6.8 | Channel-bandwidth-dependent parameter |  |
| srs-SubframeConfig | sc0 | Assuming SRS density is high (see Table 5.5.3.3-1 in TS 36.211) | FDD |
|  | sc4 | Assuming SRS density is high (see Table 5.5.3.3-2 in TS 36.211) | TDD |
| ackNackSRS-SimultaneousTransmission | TRUE | Typical value in real network |  |
| srs-MaxUpPts | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |

#### –	SoundingRS-UL-ConfigDedicated-DEFAULT

Table 4.6.3-22: SoundingRS-UL-ConfigDedicated-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SoundingRS-UL-ConfigDedicated-DEFAULT ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| srs-Bandwidth | bw0 |  |  |
|  | bw3 |  | SRSFreqHopp |
| srs-HoppingBandwidth | hbw0 |  |  |
| freqDomainPosition | 0 |  |  |
| duration | TRUE |  |  |
| srs-ConfigIndex | 20 | See Table 8.2-1 in TS 36.213 | FDD |
|  | 31 | INTEGER (0..1023) See Table 8.2-2 in TS 36.213 | TDD |
| transmissionComb | 0 |  |  |
| cyclicShift | cs0 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |
| SRSFreqHopp | Used for SRS frequency hopping. |

Table 4.6.3-22AA: Void

#### –	SoundingRS-UL-ConfigDedicatedAperiodic-r10-DEFAULT

Table 4.6.3-22AB: SoundingRS-UL-ConfigDedicatedAperiodic-r10-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SoundingRS-UL-ConfigDedicatedAperiodic-r10 ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| srs-ConfigIndexAp-r10 | 10 |  |  |
| srs-ConfigApDCI-Format4-r10 | Not present |  |  |
| srs-ActivateAp-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	SRB-ToAddModList-RECONFIG

Table 4.6.3-22A: SRB-ToAddModList-RECONFIG

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SRB-ToAddModList ::= SEQUENCE (SIZE (1..2)) OF SEQUENCE { | 2 Entries |  |  |
| srb-Identity[1] | 1 |  |  |
| rlc-Config[1] CHOICE { |  |  |  |
| explicitValue | RLC-Config-SRB-AM-RECONFIG |  |  |
| logicalChannelConfig[1] CHOICE { |  |  |  |
| defaultValue | NULL |  |  |
| } |  |  |  |
| srb-Identity[2] | 2 |  |  |
| rlc-Config[2] CHOICE { |  |  |  |
| explicitValue | RLC-Config-SRB-AM-RECONFIG |  |  |
| } |  |  |  |
| logicalChannelConfig[2] CHOICE { |  |  |  |
| defaultValue | NULL |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	SRS-TPC-PDCCH-Config-r14-DEFAULT

Table 4.6.3-22B: SRS-TPC-PDCCH-Config-r14-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SRS-TPC-PDCCH-Config-r14-DEFAULT ::= SEQUENCE { |  |  |  |
| release | NULL |  | RELEASE |
| setup SEQUENCE { |  |  | SETUP |
| srs-TPC-RNTI-r14 | '1100'H |  |  |
| startingBitOfFormat3B-r14 | 0 |  |  |
| fieldTypeFormat3B-r14 | 4 | 1 bit SRS request, 2 bit TPC command |  |
| srs-CC-SetIndexlist- | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| RELEASE | Release PDCCH configuration for SRS TPC |
| SETUP | Setup PDCCH configuration for SRS TPC |

#### –	TDD-Config-DEFAULT

Table 4.6.3-23: TDD-Config-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-Config-DEFAULT ::= SEQUENCE { |  |  |  |
| subframeAssignment | sa1 |  |  |
| specialSubframePatterns | Ssp6 |  |  |
| } |  |  |  |

#### –	TPC-PDCCH-Config-DEFAULT

Table 4.6.3-24: TPC-PDCCH-Config-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TPC-PDCCH-Config-DEFAULT ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| tpc-RNTI | ‘03FF’H |  | PUCCH |
|  | ‘01FA’H |  | PUSCH |
| tpc-Index CHOICE { |  |  |  |
| indexOfFormat3 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| PUCCH | For PUCCH |
| PUSCH | For PUSCH |

#### –	TPC-PDCCH-ConfigSCell-r13-DEFAULT

Table 4.6.3-24A: TPC-PDCCH-ConfigSCell-r13-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TPC-PDCCH-ConfigSCell-r13-DEFAULT ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| tpc-Index-PUCCH-SCell-r13 CHOICE { |  |  |  |
| indexOfFormat3 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	UplinkPowerControlCommon-DEFAULT

Table 4.6.3-25: UplinkPowerControlCommon-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlCommon-DEFAULT ::= SEQUENCE { |  |  |  |
| p0-NominalPUSCH | -85 (-85 dBm) | Typical value in real network |  |
| alpha | al08 (0.8) | Typical value in real network |  |
| p0-NominalPUCCH | -117 (-117 dBm) | Thermal noise = -121 dBm NF = 5 dB IoT = 6 dB Required SNR = -7.5 dB (1-bit A/N) -> -117 dB |  |
| deltaFList-PUCCH SEQUENCE { |  |  |  |
| deltaF-PUCCH-Format1 | deltaF0 | In accordance with RAN1 simulation results |  |
| deltaF-PUCCH-Format1b | deltaF3 | In accordance with RAN1 simulation results |  |
| deltaF-PUCCH-Format2 | deltaF0 | In accordance with RAN1 simulation results |  |
| deltaF-PUCCH-Format2a | deltaF0 | In accordance with RAN1 simulation results |  |
| deltaF-PUCCH-Format2b | deltaF0 | In accordance with RAN1 simulation results |  |
| } |  |  |  |
| deltaPreambleMsg3 | 4 |  |  |
| } |  |  |  |

#### –	UplinkPowerControlCommonSCell-r10-DEFAULT

Table 4.6.3-25A: UplinkPowerControlCommonSCell-r10-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlCommonSCell-r10 ::= SEQUENCE { |  |  |  |
| p0-NominalPUSCH-r10 | -85 (-85 dBm) |  |  |
| alpha-r10 | al08 (0.8) |  |  |
| } |  |  |  |

#### –	UplinkPowerControlCommon-v1020-DEFAULT

Table 4.6.3-25AA: UplinkPowerControlCommon-v1020-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlCommon-v1020-DEFAULT ::= SEQUENCE { |  |  |  |
| deltaF-PUCCH-Format3-r10 | deltaF0 |  | RM coding |
|  | deltaF4 |  | Dual RM coding |
| deltaF-PUCCH-Format1bCS-r10 | deltaF1 |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| RM coding | Used for Reed-Muller coding |
| Dual RM coding | Used for Dual Reed-Muller coding |

#### –	UplinkPowerControlCommon-v1530-DEFAULT

Table 4.6.3-25AAA: UplinkPowerControlCommon-v1530-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlCommon-v1530-DEFAULT ::= SEQUENCE { |  |  |  |
| deltaFList-PUCCH-r15 SEQUENCE { |  |  |  |
| deltaF-slotSPUCCH-Format1-r15 | deltaF0 |  |  |
| deltaF-slotSPUCCH-Format1a-r15 | deltaF6 |  |  |
|  | deltaF4 |  | SPUCCH-freqHopp |
| deltaF-slotSPUCCH-Format1b-r15 | deltaF8 |  |  |
|  | deltaF5 |  | SPUCCH-freqHopp |
| deltaF-slotSPUCCH-Format3-r15 | deltaF8 |  |  |
| deltaF-slotSPUCCH-RM-Format4-r15 | deltaF17 |  |  |
| deltaF-slotSPUCCH-TBCC-Format4-r15 | deltaF14 |  |  |
| deltaF-subslotSPUCCH-Format1and1a-r15 | deltaF9 |  |  |
| deltaF-subslotSPUCCH-Format1b-r15 | deltaF10 |  |  |
| deltaF-subslotSPUCCH-RM-Format4-r15 | deltaF19 |  |  |
| deltaF-subslotSPUCCH-TBCC-Format4-r15 | deltaF17 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SPUCCH-freqHopp | This condition applies when SPUCCH frequency hopping is used |

#### –	UplinkPowerControlCommonSCell-v1310-DEFAULT

Table 4.6.3-25AB: UplinkPowerControlCommonSCell-v1310-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlCommonSCell-v1310-DEFAULT ::= SEQUENCE { |  |  |  |
| p0-NominalPUCCH | -117 (-117 dBm) | Same value as in UplinkPowerControlCommon--DEFAULT used for the PCell |  |
| deltaFList-PUCCH SEQUENCE { |  |  |  |
| deltaF-PUCCH-Format1 | deltaF0 | Same value as in UplinkPowerControlCommon-DEFAULT used for the PCell |  |
| deltaF-PUCCH-Format1b | deltaF3 | Same value as in UplinkPowerControlCommon-DEFAULT used for the PCell |  |
| deltaF-PUCCH-Format2 | deltaF0 | Same value as in UplinkPowerControlCommon-DEFAULT used for the PCell |  |
| deltaF-PUCCH-Format2a | deltaF0 | Same value as in UplinkPowerControlCommon-DEFAULT used for the PCell |  |
| deltaF-PUCCH-Format2b | deltaF0 | Same value as in UplinkPowerControlCommon-DEFAULT used for the PCell |  |
| } |  |  |  |
| deltaF-PUCCH-Format3-r12 | Not present |  |  |
| deltaF-PUCCH-Format1bCS-r12 | Not present |  |  |
| deltaF-PUCCH-Format4-r13 | Not present |  |  |
| deltaF-PUCCH-Format5-13 | Not present |  |  |
| } |  |  |  |

#### –	UplinkPowerControlDedicated-DEFAULT

Table 4.6.3-26: UplinkPowerControlDedicated-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| p0-UE-PUSCH | 0 |  |  |
| deltaMCS-Enabled | en0 |  |  |
| accumulationEnabled | TRUE |  |  |
| p0-UE-PUCCH | 0 |  |  |
| pSRS-Offset | 3 (-6 dB) |  |  |
| filterCoefficient | fc4 |  |  |
| } |  |  |  |

#### –	UplinkPowerControlDedicated-v1020-DEFAULT

Table 4.6.3-26A: UplinkPowerControlDedicated-v1020-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlDedicated-v1020 ::= SEQUENCE { |  |  |  |
| deltaTxD-OffsetListPUCCH-r10 | Not Present |  |  |
| deltaTxD-OffsetListPUCCH-r10 SEQUENCE { |  |  | TxD |
| deltaTxD-OffsetPUCCH-Format1-r10 | dB0 |  |  |
| deltaTxD-OffsetPUCCH-Format1a1b-r10 | dB0 |  |  |
| deltaTxD-OffsetPUCCH-Format22a2b-r10 | dB0 |  |  |
| deltaTxD-OffsetPUCCH-Format3-r10 | dB0 |  |  |
| } |  |  |  |
| pSRS-OffsetAp-r10 | 0 |  | SRSAp |
|  |  |  | Not present |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| TxD | When PUCCH Transmission Diversity is performed. |
| SRSAp | Aperiodic SRS configured |

#### –	UplinkPowerControlDedicated-v1130-DEFAULT

Table 4.6.3-26BA: UplinkPowerControlDedicated-v1130-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlDedicated-v1130 -DEFAULT ::= SEQUENCE { |  |  |  |
| pSRS-Offset-v1130 | FFS |  |  |
| pSRS-OffsetAp-v1130 | FFS |  |  |
| deltaTxD-OffsetListPUCCH-v1130 = SEQUENCE { |  |  |  |
| deltaTxD-OffsetPUCCH-Format1bCS-r11 | FFS |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	UplinkPowerControlDedicated-v1250-DEFAULT

Table 4.6.3-26BB: UplinkPowerControlDedicated-v1250-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlDedicated-v1250 ::= SEQUENCE { |  |  |  |
| set2PowerControlParameter CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| tpc-SubframeSet-r12 | 0001100111 |  |  |
| p0-NominalPUSCH-SubframeSet2-r12 | -85 (-85 dBm) |  |  |
| alpha-SubframeSet2-r12 | al08 (0.8) |  |  |
| p0-UE-PUSCH-SubframeSet2-r12 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	UplinkPowerControlDedicated-v1530-DEFAULT

Table 4.6.3-26BC: UplinkPowerControlDedicated-v1530-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlDedicated-v1530 ::= SEQUENCE { |  |  |  |
| alpha-UE-r15 | al0 |  |  |
| p0-UE-PUSCH-r15 | 1 |  |  |
| } |  |  |  |

#### –	UplinkPowerControlDedicatedSCell-r10-DEFAULT

Table 4.6.3-26B: UplinkPowerControlDedicatedSCell-r10-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlDedicatedSCell-r10 ::= SEQUENCE { |  |  |  |
| p0-UE-PUSCH-r10 | 0 |  |  |
| deltaMCS-Enabled-r10 | en0 |  |  |
| accumulationEnabled-r10 | TRUE |  |  |
| pSRS-Offset-r10 | 7 |  |  |
| pSRS-OffsetAp-r10 | 7 |  |  |
| filterCoefficient-r10 | fc4 |  |  |
| pathlossReferenceLinking-r10 | sCell |  |  |
| } |  |  |  |

#### –	UplinkPowerControlDedicatedSCell-v1310-DEFAULT

Table 4.6.3-26C: UplinkPowerControlDedicatedSCell-v1310-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlDedicatedSCell-v1310 ::= SEQUENCE { |  |  |  |
| p0-UE-PUCCH | 0 |  |  |
| deltaTxD-OffsetListPUCCH-r10 | Not present |  |  |
| } |  |  |  |

#### -	RadioResourceConfigDedicated-DRB-Mod

Table 4.6.3-27: RadioResourceConfigDedicated-DRB-Mod

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-DRB ::= SEQUENCE { |  |  |  |  |
| srb-ToAddModList |  | Not present |  |  |
| drb-ToAddModList |  | DRB-ToAddModList-RECONFIG |  |  |
| } |  |  |  |  |
| drb-ToReleaseList |  | Not present |  |  |
| mac-MainConfig |  | Not present |  |  |
| sps-Config |  | Not present |  |  |
| physicalConfigDedicated |  | Not present |  |  |
| } |  |  |  |  |

#### -	RadioResourceConfigDedicated-PCell-PATTERN

Table 4.6.3-28: RadioResourceConfigDedicated-PCell-PATTERN

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| measSubframePatternPCell ::= CHOICE { |  |  |  |  |
| MeasSubframePattern-r10 ::= CHOICE { |  |  |  |  |
| subframePatternFDD-r10 |  | ‘0011001100110011001100110011001100110011’B |  |  |
| subframePatternTDD-r10 |  | FFS |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### -	OtherConfig-r9

Table 4.6.3-29: OtherConfig-r9

| Derivation Path: 36.331 clause 6.3.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| OtherConfig-r9 ::= SEQUENCE { |  |  |  |
| reportProximityConfig-r9 ::= SEQUENCE { |  |  |  |
| proximityIndicationEUTRA-r9 | Not Present |  |  |
| proximityIndicationUTRA-r9 | Not Present |  |  |
| } |  |  |  |
| IDC-Config-r11 SEQUENCE { |  |  |  |
| idc-Indication-r11 | Not Present |  |  |
| autonomouseDenialParameters-r11 ::= SEQUENCE { |  |  |  |
| autonomousDenialSubframes-r11 | Not Present |  |  |
| autonomousDenialValidity-r11 | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| PowerPrefIndicationConfig-r11 CHOICE { |  |  |  |
| Release | NULL |  | Release |
| setup SEQUENCE { |  |  | Setup |
| powerPrefIndicationTimer-r11 | s30 |  |  |
| } |  |  |  |
| } |  |  |  |
| delayBudgetReportingConfig-r14 CHOICE{ |  |  | eVoLTE |
| Release | NULL |  | Release |
| setup SEQUENCE{ |  |  | Setup |
| delayBudgetReportingProhibitTimer-r14 | s0 |  |  |
| } |  |  |  |
| } |  |  |  |
| ObtainLocationConfig-r11 ::= SEQUENCE { |  |  |  |
| obtainLocation-r11 | Not present |  |  |
| } |  |  |  |
| sps-AssistanceInfoReport-r14 | Not present |  |  |
| measConfigAppLayer-r15 | Not present |  |  |
| measConfigAppLayer-r15 :: = CHOICE { |  |  | QMC |
| Release | NULL |  |  |
| setup SEQUENCE { |  |  |  |
| measConfigAppLayerContainer-r15 | Octets string (1…1000) |  |  |
| serviceType | qoe |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| eVoLTE | The UE is configured to setup or release delayBudgetReport in case of VoLTE enhancement |
| QMC | The UE is configured to setup or release application layer QoE measurement report |
| Setup | The UE is allowed to send power preference indications or send delaybudgetreport. |
| Release | The UE is not allowed to send power preference indications or send delay budgetreport. |

#### -	WLAN-OffloadConfig-r12

Table 4.6.3-30: WLAN-OffloadConfig-DEFAULT

| Derivation Path: 36.331 clause 6.3.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| WLAN-OffloadConfig-r12::= SEQUENCE { |  |  |  |
| thresholdRSRP-r12 | Not Present |  |  |
| thresholdRSRP-r12::= SEQUENCE { |  |  | RSRP |
| thresholdRSRP-Low-r12 | Set according to specific message content |  |  |
| thresholdRSRP-High-r12 | Set according to specific message content |  |  |
| } |  |  |  |
| thresholdRSRQ-r12 | Not Present |  | RSRQ |
| thresholdRSRQ-r12::= SEQUENCE { |  |  |  |
| thresholdRSRQ-Low-r12 | Set according to specific message content |  |  |
| thresholdRSRQ-High-r12 | Set according to specific message content |  |  |
| } |  |  |  |
| thresholdRSRQ-OnAllSymbolsWithWB-r12 | Not Present |  |  |
| thresholdRSRQ-OnAllSymbols-r12 | Not Present |  |  |
| thresholdRSRQ-WB-r12 | Not Present |  |  |
| thresholdChannelUtilization-r12 | Not Present |  |  |
| thresholdChannelUtilization-r12::= SEQUENCE { |  |  | ChanUtilization |
| thresholdChannelUtilizationLow-r12 | Set according to specific message content |  |  |
| thresholdChannelUtilizationHigh-r12 | Set according to specific message content |  |  |
| } |  |  |  |
| thresholdBackhaul-Bandwidth-r12 | Not Present |  |  |
| thresholdBackhaul-Bandwidth-r12::= SEQUENCE { |  |  | BackhaulBW |
| thresholdBackhaulDL-BandwidthLow-r12 | Set according to specific message content |  |  |
| thresholdBackhaulDL-BandwidthHigh-r12 | Set according to specific message content |  |  |
| thresholdBackhaulUL-BandwidthLow-r12 | Set according to specific message content |  |  |
| thresholdBackhaulUL-BandwidthHigh-r12 | Set according to specific message content |  |  |
| } |  |  |  |
| thresholdBeaconRSSI-r12 | Not Present |  |  |
| thresholdBeaconRSSI-r12::= SEQUENCE { |  |  | BeaconRSSI |
| thresholdBeaconRSSI-Low-r12 | Set according to specific message content |  |  |
| thresholdBeaconRSSI-High-r12 | Set according to specific message content |  |  |
| } |  |  |  |
| offloadPreferenceIndicator-r12 | ‘1111 1111 1111 1111’B | All IP flows enabled |  |
| t-SteeringWLAN-r12 | 0 | Typical value in real networks |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| RSRP | If RSRP based thresholds are to be used |
| RSRQ | If RSRQ based thresholds are to be used |
| ChanUtilization | If Channel Utilization based thresholds are to be used |
| BackhaulBW | If Backhaul bandwidth thresholds are to be used |
| BeaconRSSI | If Beacon RSSI Thresholds are to be used |

#### –	EIMTA-MainConfig-r12-DEFAULT

Table 4.6.3-31: EIMTA-MainConfig-r12-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EIMTA-MainConfig-r12 ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| eimta-RNTI-r12 | Set to the value of the C-RNTI of the UE |  |  |
| eimta-CommandPeriodicity-r12 | sf10 |  |  |
| eimta-CommandSubframeSet-r12 | 1111111111 |  | FDD |
|  | 1100011000 |  | TDD |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |

#### –	EIMTA-MainConfigServCell-r12-DEFAULT

Table 4.6.3-32: EIMTA-MainConfigServCell-r12-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EIMTA-MainConfigServCell-r12 ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| eimta-UL-DL-ConfigIndex-r12 | 1 |  |  |
| eimta-HARQ-ReferenceConfig-r12 | Sa2 |  |  |
| mbsfn-SubframeConfigList-v1250 | Not present |  |  |
| mbsfn-SubframeConfigList-v1250 ::= CHOICE { |  |  | MBMS |
| setup SEQUENCE { |  |  |  |
| mbsfn-SubframeConfigList | Not present |  |  |
| mbsfn-SubframeConfigList SEQUENCE (SIZE (1..maxMBSFN-Allocations)) OF SEQUENCE { |  |  | MBMS |
| radioframeAllocationPeriod | n4 |  |  |
| radioframeAllocationOffset | 1 |  | FDD |
|  | 0 |  | TDD |
| subframeAllocation CHOICE{ |  |  |  |
| oneFrame | ‘100000’ B |  | FDD |
|  | ‘000010’ B |  | TDD |
| fourFrames | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| MBMS | MBMS cell environment |
| FDD | FDD cell environment |
| TDD | TDD cell environment |

#### –	LWIP-Configuration-GENERIC

Table 4.6.3-33: LWIP-Configuration-GENERIC (SSID)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| LWIP-Configuration-r13 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| lwip-MobilityConfig-r13 SEQUENCE { |  |  |  |
| wlan-ToReleaseList-r13 OF SEQUENCE { | 1 entry |  | RELEASE |
| ssid-r12[1] | SSID |  |  |
| bssid-r12 | Not Present |  |  |
| hessid-r12 | Not Present |  |  |
| } |  |  |  |
| wlan-ToAddList-r13 OF SEQUENCE { | 1 entry |  | ADD |
| ssid-r12[1] | SSID |  |  |
| bssid-r12 | Not Present |  |  |
| hessid-r12 | Not Present |  |  |
| } |  |  |  |
| associationTimer-r13 | s10 | 10 seconds | SUCCESS |
| successReportRequested-r13 | true |  | SUCCESS |
| } |  |  |  |
| TunnelConfigLWIP-r13 SEQUENCE { |  |  |  |
| ip-Address-r13 CHOICE { |  |  |  |
| ipv4-r13 | [IP address of the LWIP-SeGW, reachable from the WLAN side] |  |  |
| ipv6-r13 | [IP address of the LWIP-SeGW, reachable from the WLAN side] |  |  |
| } |  |  |  |
| ike-Identity-r13 SEQUENCE { |  |  |  |
| idI-r13 | [IKE Identity] |  |  |
| } |  |  |  |
| lwip-Counter-r13 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| ADD | Add a WLAN entry to the mobility configuration |
| RELEASE | Release a WLAN entry from the mobility configuration |
| SUCCESS | Request a WLAN status report after association |

#### –	LWA-Configuration-GENERIC

Table 4.6.3-34: LWA-Configuration-GENERIC (SSID)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| LWA-Configuration-r13 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| lwa-MobilityConfig-r13 SEQUENCE { |  |  |  |
| wlan-ToReleaseList-r13 OF SEQUENCE { | 1 entry |  | RELEASE |
| ssid-r12[1] | SSID |  |  |
| bssid-r12 | Not Present |  |  |
| hessid-r12 | Not Present |  |  |
| } |  |  |  |
| wlan-ToAddList-r13 OF SEQUENCE { | 1 entry |  | ADD |
| ssid-r12[1] | SSID |  |  |
| bssid-r12 | Not Present |  |  |
| hessid-r12 | Not Present |  |  |
| } |  |  |  |
| associationTimer-r13 | s10 | 10 seconds | SUCCESS |
| successReportRequested-r13 | true |  | SUCCESS |
| } |  |  |  |
| lwa-WT-Counter-r13 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| ADD | Add a WLAN entry to the mobility configuration |
| RELEASE | Release a WLAN entry from the mobility configuration |
| SUCCESS | Request a WLAN status report after association |

#### –	LWIP-Configuration-W2-GENERIC

Table 4.6.3-35: LWIP-Configuration-W2-GENERIC (SSID1, SSID2)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| LWIP-Configuration-r13 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| lwip-MobilityConfig-r13 SEQUENCE { |  |  |  |
| wlan-ToReleaseList-r13 OF SEQUENCE { | 1 entry |  |  |
| ssid-r12[1] | SSID1 |  |  |
| bssid-r12 | Not Present |  |  |
| hessid-r12 | Not Present |  |  |
| } |  |  |  |
| wlan-ToAddList-r13 OF SEQUENCE { | 1 entry |  |  |
| ssid-r12[1] | SSID2 |  |  |
| bssid-r12 | Not Present |  |  |
| hessid-r12 | Not Present |  |  |
| } |  |  |  |
| associationTimer-r13 | s10 | 10 seconds | SUCCESS |
| successReportRequested-r13 | true |  | SUCCESS |
| } |  |  |  |
| TunnelConfigLWIP-r13 SEQUENCE { |  |  |  |
| ip-Address-r13 CHOICE { |  |  |  |
| ipv4-r13 | [IP address of the LWIP-SeGW, reachable from the WLAN side] |  |  |
| ipv6-r13 | [IP address of the LWIP-SeGW, reachable from the WLAN side] |  |  |
| } |  |  |  |
| ike-Identity-r13 SEQUENCE { |  |  |  |
| idI-r13 | [IKE Identity] |  |  |
| } |  |  |  |
| lwip-Counter-r13 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SUCCESS | Request a WLAN status report after association |

#### –	LWA-Configuration-W2-GENERIC

Table 4.6.3-36: LWA-Configuration-W2-GENERIC (SSID1, SSID2)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| LWA-Configuration-r13 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| lwa-MobilityConfig-r13 SEQUENCE { |  |  |  |
| wlan-ToReleaseList-r13 OF SEQUENCE { | 1 entry |  |  |
| ssid-r12[1] | SSID1 |  |  |
| bssid-r12 | Not Present |  |  |
| hessid-r12 | Not Present |  |  |
| } |  |  |  |
| wlan-ToAddList-r13 OF SEQUENCE { | 1 entry |  |  |
| ssid-r12[1] | SSID2 |  |  |
| bssid-r12 | Not Present |  |  |
| hessid-r12 | Not Present |  |  |
| } |  |  |  |
| associationTimer-r13 | s10 | 10 seconds | SUCCESS |
| successReportRequested-r13 | true |  | SUCCESS |
| } |  |  |  |
| lwa-WT-Counter-r13 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SUCCESS | Request a WLAN status report after association |

### 4.6.4	Security control information elements

#### –	SecurityConfigHO-DEFAULT

Table 4.6.4-1: SecurityConfigHO-DEFAULT

| Derivation Path: 36.331 clause 6.2,2, 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SecurityConfigHO-DEFAULT ::= SEQUENCE { |  |  |  |
| handoverType CHOICE { |  |  |  |
| intraLTE SEQUENCE { |  |  |  |
| securityAlgorithmConfig | Not present |  |  |
| keyChangeIndicator | FALSE |  |  |
| nextHopChainingCount | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	SecurityConfigSMC-DEFAULT

Table 4.6.4-2: SecurityConfigSMC-DEFAULT

| Derivation Path: 36.331 clause 6.2,2, 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SecurityConfigSMC-DEFAULT ::= SEQUENCE { |  |  |  |  |
| securityAlgorithmConfig SEQUENCE { |  |  |  |  |
| cipheringAlgorithm |  | Set according to PIXIT parameter for default ciphering algorithm |  | For SIG |
|  |  | eea0 |  | For RF |
| integrityProtAlgorithm |  | Set according to PIXIT parameter for default integrity protection algorithm |  | For SIG |
|  |  | spare1 or eia0-v920 | This IE is set to one of the algorithms supported by the UE as indicated in the IE "UE network capability" in the ATTACH REQUEST message. | For RF |
|  |  | eia1 |  |  |
|  |  | eia2 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| For SIG | Used for signalling test cases |
| For RF | Used for RF/RRM test cases |

### 4.6.5	Mobility control information elements

#### –	MobilityControlInfo-HO

Table 4.6.5-1: MobilityControlInfo-HO

| Derivation Path: 36.331 clause 6.3.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo-HO ::= SEQUENCE { |  |  |  |
| targetPhysCellId | Set according to specific message content |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Set according to specific message content |  |  |
| ul-CarrierFreq | Set according to specific message content |  | FDD |
| ul-CarrierFreq | Not present |  | TDD |
| } |  |  |  |
| carrierFreq SEQUENCE { } | Not present |  | Band > 64 |
| carrierBandwidth | Not present |  | SAME-BW |
| carrierBandwidth SEQUENCE { |  |  | DIFF-BW |
| dl-Bandwidth | Same downlink bandwidth as used for target cell |  |  |
| ul-Bandwidth | Not present |  | TDD |
| ul-Bandwidth | Same uplink bandwidth as used for target cell |  | FDD and Asymmetric-BW |
| } |  |  |  |
| additionalSpectrumEmission | Not present |  |  |
|  | 1 |  | HO-TO-EUTRA，HO-TO-EN-DC |
| t304 | ms1000 |  |  |
| newUE-Identity | SS arbitrarily selects a value between ‘003C’H and ‘FFF2’H. |  |  |
| radioResourceConfigCommon | RadioResourceConfigCommon-DEFAULT |  |  |
| rach-ConfigDedicated | Not present |  |  |
|  | Rach-ConfigDedicated-DEFAULT |  | FullConfig, HO-TO-EUTRA，HO-TO-EN-DC |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Set according to specific message content |  |  |
| } |  |  |  |
| mobilityControlInfoV2X-r14 SEQUENCE { |  |  | HO-V2X |
| v2x-CommTxPoolExceptional-r14 SEQUENCE { |  |  |  |
| sl-OffsetIndicator-r14 | 0 | small-r12 |  |
| sl-Subframe-r14 | 00000000 11110000 0000 | bs20-r14 |  |
| adjacencyPSCCH-PSSCH-r14 | TRUE | BOOLEAN |  |
| sizeSubchannel-r14 | n5 | ENUMERATED {n5} | BW10 |
|  | n10 | ENUMERATED {n10} | BW20 |
| numSubchannel-r14 | n5 | ENUMERATED {n5} |  |
| startRB-Subchannel-r14 | 0 |  |  |
| startRB-PSCCH-Pool-r14 | Not present |  |  |
| rxParametersNCell-r14 | Not present |  |  |
| dataTxParameters-r14 SEQUENCE { |  |  |  |
| alpha-r12 | al0 |  |  |
| p0-r12 | 31 |  |  |
| } |  |  |  |
| zoneID-r14 | Not present |  |  |
| } |  |  |  |
| v2x-CommRxPool-r14 SEQUENCE { |  |  |  |
| sl-OffsetIndicator-r14 | 0 | small-r12 |  |
| sl-Subframe-r14 | 00001111 00000000 0000 | bs20-r14 |  |
| adjacencyPSCCH-PSSCH-r14 | TRUE | BOOLEAN |  |
| sizeSubchannel-r14 | n5 | ENUMERATED {n5} | BW10 |
|  | n10 | ENUMERATED {n10} | BW20 |
| numSubchannel-r14 | n5 | ENUMERATED {n5} |  |
| startRB-Subchannel-r14 | 0 |  |  |
| startRB-PSCCH-Pool-r14 | Not present |  |  |
| rxParametersNCell-r14 | Not present |  |  |
| dataTxParameters-r14 | Not present |  |  |
| zoneID-r14 | Not present |  |  |
| } |  |  |  |
| v2x-CommSyncConfig-r14 | Not present |  |  |
| cbr-MobilityTxConfigList-r14 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SAME-BW | Source and target cell are configured with same bandwidth |
| DIFF-BW | Source and target cell are configured with different bandwidth |
| Band > 64 | If band > 64 is selected |
| HO-V2X | V2X sidelink communication handover |
| BW10 | 10 MHz channel bandwidth for V2X communication |
| BW20 | 20 MHz channel bandwidth for V2X communication |
| FDD | FDD cell environment |
| TDD | TDD cell environment |
| HO-TO-EUTRA | Inter-RAT handover to E-UTRA |
| Asymmetric-BW | Target cell is configured with different dl-bandwidth and ul-bandwidth |
| HO-TO-EN-DC | Inter-RAT handover to EN-DC |

### 4.6.6	Measurement information elements

#### -	FreqBandIndicatorNR-r15

Table 4.6.6-0: FreqBandIndicatorNR-r15

| Derivation Path: TS 36.331 [17], clause 6.3.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FreqBandIndicatorNR-r15 | NR Operating band under test |  |  |

#### -	MeasConfig-DEFAULT

Table 4.6.6-1: MeasConfig-DEFAULT

| Derivation Path: 36.331, clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| measObjectToRemoveList | Not present |  |  |
| measObjectToAddModList | Not present |  |  |
|  | MeasObjectEUTRA-GENERIC |  | RF |
| reportConfigToRemoveList | Not present |  |  |
| reportConfigToAddModList | Not present |  |  |
|  | ReportConfigToAddModList_DEFAULT |  | RF |
| measIdToRemoveList | Not present |  |  |
| measIdToAddModList | Not present |  |  |
|  | MeasIdToAddModList_DEFAULT |  | RF |
| quantityConfig | QuantityConfig-DEFAULT |  |  |
| measGapConfig | Not present |  |  |
|  | MeasGapConfig-GP1 |  | INTER-FREQ, UTRAN |
|  | MeasGapConfig-GP2 |  | GERAN, INTER-RAT |
|  | MeasGapConfig-CE |  | intraFreq-CE-NeedForGaps |
| s-Measure | Not present |  |  |
| preRegistrationInfoHRPD | Not present |  |  |
| speedStatePars | Not present |  |  |
| measObjectToAddModList-v9e0 SEQUENCE { |  |  | Band > 64 and RF |
| measObjectEUTRA-v9e0 SEQUENCE { | N entries | N needs to be the same number of entries as in measObjectToAddModList |  |
| carrierFreq-v9e0[n] | Downlink EARFCN for Freq |  |  |
| } |  |  |  |
| } |  |  |  |
| measScaleFactor-r12 | sf-EUTRA-cf1 | specifies the factor for scaling the measurement performance requirements in TS 36.133 | IncMon |
| heightThreshRef-r15 | Not present |  |  |
| } |  |  |  |
| Note: For signalling test, table 6.5.1-1 is applied. |  |  |  |

| Condition | Explanation |
| --- | --- |
| INTER-FREQ | For E-UTRA inter-freq measurements |
| INTER-RAT | For inter-RAT measurements with UTRAN and GERAN |
| Band > 64 | If band > 64 is selected |
| intraFreq-CE-NeedForGaps | For UE having set pc_intraFreq-CE-NeedForGaps to TRUE. |
| RF | For RF, performance and RRM testing |
| IncMon | For IncMon RRM testing |

#### -	MeasGapConfig-GP1

Table 4.6.6-1A: MeasGapConfig-GP1

| Derivation Path: 36.331, clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapConfig-GP1 ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| gapOffset CHOICE { |  |  |  |
| gp0 | 0 | TGRP = 40 ms |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### -	MeasGapConfig-CE

Table 4.6.6-1BA: MeasGapConfig-CE

| Derivation Path: 36.331, clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapConfig-CE ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| gapOffset CHOICE { |  |  |  |
| gp0 | 20 | TGRP = 40 ms |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### -	MeasGapConfig-GP2

Table 4.6.6-1B: MeasGapConfig-GP2

| Derivation Path: 36.331, clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapConfig-GP2 ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| gapOffset CHOICE { |  |  |  |
| gp1 | 0 | TGRP = 80 ms |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### -	MeasDS-Config-DEFAULT

Table 4.6.6-1CA: MeasDS-Config-DEFAULT

| Derivation Path: 36.331, clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasDS-Config-DEFAULT ::= CHOICE { |  |  |  |
| release | NULL |  |  |
| setup SEQUENCE { |  |  |  |
| dmtc-PeriodOffset-r12 CHOICE { |  |  |  |
| ms160-r12 | 10 |  |  |
| } |  |  |  |
| ds-OccasionDuration-r12 CHOICE { |  |  |  |
| durationFDD-r12 | 1 |  | FDD |
| durationTDD-r12 | 2 |  | TDD |
|  | 1 |  | LAA SCell |
| } |  |  |  |
| measCSI-RS-ToRemoveList-r12 | Not present |  |  |
| measCSI-RS-ToAddModList-r12 SEQUENCE (SIZE (1..maxCSI-RS-Meas-r12)) OF MeasCSI-RS-Config-r12 { |  |  |  |
|  | Not present |  |  |
|  | MeasCSI-RS-Config-Default |  | CSI-RS |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |
| CSI-RS | For CSI-RS measurements |
| LAA SCell | For LAA SCell environment |

#### -	MeasCSI-RS-Config-DEFAULT

Table 4.6.6-1CB: MeasCSI-RS-Config-DEFAULT

| Derivation Path: 36.331, clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasCSI-RS-Config-DEFAULT ::= SEQUENCE { |  |  |  |
| measCSI-RS-Id-r12 | Set according to specific message content |  |  |
| physCellId-r12 | Set according to specific message content |  |  |
| scramblingIdentity-r12 | 1 |  |  |
| resourceConfig-r12 | 2 |  |  |
| subframeOffset-r12 | 0 |  |  |
| csi-RS-IndividualOffset-r12 | 0 |  |  |
| } |  |  |  |

#### –	MeasObjectCDMA2000-GENERIC

Table 4.6.6-1C: MeasObjectCDMA2000-GENERIC

| Derivation Path: 36.331 clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectCDMA2000-GENERIC ::= SEQUENCE { |  |  |  |
| cdma2000-Type | As per specific message content | [type1XRTT, typeHRPD] |  |
| carrierFreq SEQUENCE { |  |  |  |
| bandClass | As per specific message content | Should be one of the following: bc0, bc1, bc2, bc3, bc4, bc5, bc6, bc7, bc8, bc9, bc10, bc11, bc12, bc13, bc14, bc15, bc16, bc17, spare14, spare13, spare12, spare11, spare10,spare9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1, ... |  |
| arfcn | Downlink channel number | INTEGER (0 to 2047) |  |
| } |  |  |  |
| searchWindowSize | As per specific message content | INTEGER (0..15) |  |
| offsetFreq | 0 (0 dB) |  |  |
| cellsToRemoveList | Not present |  |  |
| cellsToAddModList | [Not present] |  |  |
| cellForWhichToReportCGI | Not present |  |  |
| } |  |  |  |

#### –	ReportConfigToAddModList_DEFAULT

Table 4.6.6-1D: ReportConfigToAddModList_DEFAULT

| Derivation Path: 36.331, clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigToAddModList_DEFAULT ::= SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |

#### –	MeasIdToAddModList_DEFAULT

Table 4.6.6-1E: MeasIdToAddModList_DEFAULT

| Derivation Path: 36.331, clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasIdToAddModList_DEFAULT ::= SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |

#### -	MeasObjectEUTRA-GENERIC

Table 4.6.6-2: MeasObjectEUTRA-GENERIC(Freq)

| Derivation Path: 36.331 clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectEUTRA-GENERIC(Freq) ::= SEQUENCE { |  |  |  |
| carrierFreq | Downlink EARFCN for Freq |  |  |
| carrierFreq | 65535 | maxEARFCN | Band > 64 |
| allowedmeasBandwidth | The number of the resource blocks for Freq |  |  |
| presenceAntennaPort1 | FALSE |  |  |
|  | TRUE | At least two cell-specific antenna ports are used in all neighbouring cells. | All neighCells with port1 |
| neighbourCellConfig | '01'B (No MBSFN subframes are present in all neighbour cells) | MBSFN doesn’t apply by default. |  |
| offsetFreq | 0 (dB 0) |  |  |
| cellsToRemoveList | Not present |  |  |
| cellsToAddModList | Not present |  |  |
| blackCellsToRemoveList | Not present |  |  |
| blackCellsToAddModList | Not present |  |  |
| cellForWhichToReportCGI | Not present |  |  |
| measCycleSCell-r10 | sf512 |  | SCell_EXIST |
| measSubframePatternConfigNeigh-r10 | Not present |  |  |
| measDS-Config-r12 | MeasDS-Config-DEFAULT |  | DS_Meas |
| reducedMeasPerformance-r12 | TRUE | For reduced measurement performance, otherwise it is normal measurement performance | IncMon |
| rmtc-Config-r13 | Not present |  |  |
| tx-ResourcePoolToRemoveList-r14 | Not present |  |  |
| tx-ResourcePoolToAddList-r14 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SCell_EXIST | When SCell exists on the carrier frequency indicated by carrierFreq |
| All neighCells with port1 | Used for all neighbouring cells with at least two cell-specific antenna ports |
| DS_Meas | Used for discovery signals measurement on the carrier frequency indicated by carrierFreq |
| Band > 64 | If band > 64 is selected |
| IncMon | For IncMon RRM testing |

#### -	MeasObjectGERAN-GENERIC

Table 4.6.6-2A: MeasObjectGERAN-GENERIC(Freq)

| Derivation Path: 36.331 clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectGERAN-GENERIC(Freq) ::= SEQUENCE { |  |  |  |
| carrierFreqs SEQUENCE { |  |  |  |
| startingARFCN | Downlink GERAN ARFCN of Freq |  |  |
| bandIndicator | Set according to the band used for GERAN cells under test |  |  |
| followingARFCNs CHOICE { |  |  |  |
| explicitListOfARFCNs | Set the corresponding ARFCN of GERAN cells under test |  |  |
| } |  |  |  |
| } |  |  |  |
| offsetFreq | 0 (dB 0) |  |  |
| ncc-Permitted | ‘01000000'B | NCC=1 permitted |  |
| cellForWhichToReportCGI | Not present |  |  |
| } |  |  |  |

#### -	MeasObjectNR-GENERIC

Table 4.6.6-2B: MeasObjectNR-GENERIC (Freq)

| Derivation Path: 36.331 clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR-GENERIC(Freq) ::= SEQUENCE { |  |  |  |
| carrierFreq-r15 | Downlink ARFCN of ssbFrequency |  |  |
| rs-ConfigSSB-r15 ::= SEQUENCE { |  |  |  |
| measTimingConfig-r15 ::= SEQUENCE { |  |  |  |
| periodicityAndOffset-r15 CHOICE { |  |  |  |
| sf20-r15 | 0 |  |  |
| } |  |  |  |
| ssb-Duration-r15 | sf5 |  |  |
| } |  |  |  |
| subcarrierSpacingSSB-r15 | kHz15 |  | FR1_FDD |
|  | kHz30 |  | FR1_TDD |
|  | kHz120 |  | FR2_TDD |
| } |  |  |  |
| threshRS-Index-r15 | Not present |  |  |
| maxRS-IndexCellQual-r15 | Not present |  |  |
| offsetFreq-r15 | 0 |  |  |
| blackCellsToRemoveList-r15 | Not present |  |  |
| blackCellsToAddModList-r15 | Not present |  |  |
| quantityConfigSet-r15 | 1 |  |  |
| cellsForWhichToReportSFTD-r15 | Not present |  |  |
| cellForWhichToReportCGI-r15 | Not present |  |  |
| deriveSSB-IndexFromCell-r15 | Not present |  |  |
| ss-RSSI-Measurement-r15 | Not present |  |  |
| bandNR-r15 CHOICE { |  |  |  |
| Setup | FreqBandIndicatorNR-r15 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FR1_FDD | FDD frequency range < 6GHz |
| FR1_TDD | TDD frequency range < 6GHz |
| FR2_TDD | TDD frequency range > 6GHz |

#### -	MeasObjectUTRA-GENERIC

Table 4.6.6-3: MeasObjectUTRA-GENERIC(Freq)

| Derivation Path: 36.331 clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectUTRA-GENERIC(Freq) ::= SEQUENCE { |  |  |  |
| carrierFreq | Downlink UARFCN of Freq |  |  |
| offsetFreq | 0 (dB 0) |  |  |
| cellsToRemoveList | Not present |  |  |
| cellsToAddModList | Not present | For UTRA, the neighbouring cell list needs to be provided in specific test cases. |  |
| cellForWhichToReportCGI | Not present |  |  |
| reducedMeasPerformance-r12 | TRUE | For reduced measurement performance, otherwise it is normal measurement performance | IncMon |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| IncMon | For IncMon RRM testing |

#### –	MeasObjectWLAN-W2-GENERIC

Table 4.6.6-3AB: MeasObjectWLAN-W2-GENERIC (Freq, SSID1, SSID2)

| Derivation Path: 36.331 clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measObjectWLAN-r13 SEQUENCE { |  |  |  |
| carrierFreq-r13 | Freq | WLAN band and carrier as used in test setup for 2 entries |  |
| wlan-ToAddModList-r13 OF SEQUENCE { |  | 2 entries |  |
| ssid-r12[1] | SSID1 |  |  |
| ssid-r12[2] | SSID2 |  |  |
| } |  |  |  |
| wlan-ToRemoveList-r13 | Not Present |  |  |
| } |  |  |  |

#### -	QuantityConfig-DEFAULT

Table 4.6.6-3A: QuantityConfig-DEFAULT

| Derivation Path: 36.331, clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QuantityConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| quantityConfigEUTRA SEQUENCE { |  |  |  |
| filterCoefficientRSRP | Not present | DEFAULT fc4 |  |
| filterCoefficientRSRQ | Not present | DEFAULT fc4 |  |
| } |  |  |  |
| quantityConfigUTRA SEQUENCE {} | Not present |  |  |
| quantityConfigUTRA SEQUENCE { |  |  | UTRAN |
| measQuantityUTRA-FDD | cpich-EcN0 |  |  |
| measQuantityUTRA-FDD | cpich-RSCP | For signalling test cases |  |
| measQuantityUTRA-TDD | pccpch-RSCP |  |  |
| filterCoefficient | Not present | DEFAULT fc4 |  |
| } |  |  |  |
| quantityConfigGERAN SEQUENCE {} | Not present |  |  |
| quantityConfigGERAN SEQUENCE { |  |  | GERAN |
| measQuantityGERAN | rssi |  |  |
| filterCoefficient | Not present | DEFAULT fc2 |  |
| } |  |  |  |
| quantityConfigCDMA2000 SEQUENCE {} | Not present |  |  |
| quantityConfigCDMA2000 SEQUENCE { |  |  | CDMA2000 |
| measQuantityCDMA2000 | [FFS] |  |  |
| } |  |  |  |
| quantityConfigEUTRA-v1250 SEQUENCE { |  |  |  |
| filterCoefficientCSI-RSRP-r12 | Not present | DEFAULT fc4 | CSI-RSRP |
| } |  |  |  |
| quantityConfigEUTRA-v1310 | Not present |  |  |
| quantityConfigEUTRA-v1310 SEQUENCE { |  |  | RS-SINR |
| filterCoefficientRS-SINR-r13 | Not present | DEFAULT fc4 |  |
| } |  |  |  |
| quantityConfigWLAN-r13 | Not present |  |  |
| quantityConfigWLAN-r13 SEQUENCE { |  |  | WLAN |
| measQuantityWLAN-r13 | rssiWLAN |  |  |
| filterCoefficientRS-SINR-r13 | Not present | DEFAULT fc4 |  |
| } |  |  |  |
| quantityConfigNRList-r15 | Not present |  |  |
| quantityConfigNRList-r15 (SIZE (1..maxQuantSetsNR-r15)) OF SEQUENCE { | 2 entries |  | NR |
| measQuantityCellNR-r15[1] SEQUENCE { |  |  |  |
| filterCoeff-RSRP-r15 | Not present | DEFAULT fc4 |  |
| filterCoeff-RSRQ-r15 | Not present | DEFAULT fc4 |  |
| filterCoefficient-SINR-r13 | Not present | DEFAULT fc4 |  |
| } |  |  |  |
| measQuantityRS-IndexNR-r15[1] | Not present |  |  |
| measQuantityRS-IndexNR-r15[1] SEQUENCE { |  |  | SSB_INDEX |
| filterCoeff-RSRP-r15 | Not present | DEFAULT fc4 |  |
| filterCoeff-RSRQ-r15 | Not present | DEFAULT fc4 |  |
| filterCoefficient-SINR-r13 | Not present | DEFAULT fc4 |  |
| } |  |  |  |
| measQuantityCellNR-r15[2] SEQUENCE { |  |  |  |
| filterCoeff-RSRP-r15 | Not present | DEFAULT fc4 |  |
| filterCoeff-RSRQ-r15 | Not present | DEFAULT fc4 |  |
| filterCoefficient-SINR-r13 | Not present | DEFAULT fc4 |  |
| } |  |  |  |
| measQuantityRS-IndexNR-r15[2] | Not present |  |  |
| measQuantityRS-IndexNR-r15[2] SEQUENCE { |  |  | SSB_INDEX |
| filterCoeff-RSRP-r15 | Not present | DEFAULT fc4 |  |
| filterCoeff-RSRQ-r15 | Not present | DEFAULT fc4 |  |
| filterCoefficient-SINR-r13 | Not present | DEFAULT fc4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRAN | For inter-RAT measurements with UTRAN |
| GERAN | For inter-RAT measurements with GERAN |
| CDMA2000 | For inter-RAT measurements with CDMA2000 |
| CSI-RSRP | For CSI-RSRP measurements |
| RS-SINR | For RS-SINR measurements |
| WLAN | For inter-RAT measurements with WLAN |
| NR | For inter-RAT measurements with NR |
| SSB_INDEX | SSB index is required to be included in measurement report. |

#### -	ReportConfigEUTRA-A1

Table 4.6.6-4: ReportConfigEUTRA-A1(Thres)

| Derivation Path: 36.331 clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigEUTRA-A1(Thres) ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA1 SEQUENCE { |  |  |  |
| a1-Threshold CHOICE { |  |  |  |
| threshold-RSRP | Thres+140 | Thres is actual threshold value in dBm |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| hysteresis | 0 (0 dB) |  |  |
| timeToTrigger | ms256 |  |  |
| } |  |  |  |
| } |  |  |  |
| triggerQuantity | rsrp |  |  |
| reportQuantity | both |  |  |
| maxReportCells | 1 |  |  |
| reportInterval | ms1024 |  |  |
| reportAmount | r1 |  |  |
| } |  |  |  |

#### -	ReportConfigEUTRA-A2

Table 4.6.6-5: ReportConfigEUTRA-A2(Thres)

| Derivation Path: 36.331 clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigEUTRA-A2(Thres) ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA2 SEQUENCE { |  |  |  |
| a2-Threshold CHOICE { |  |  |  |
| threshold-RSRP | Thres+140 | Thres is actual threshold value in dBm |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| hysteresis | 0 (0 dB) |  |  |
| timeToTrigger | ms320 |  |  |
| } |  |  |  |
| } |  |  |  |
| triggerQuantity | rsrp |  |  |
| reportQuantity | both |  |  |
| maxReportCells | 1 |  |  |
| reportInterval | ms1024 |  |  |
| reportAmount | r1 |  |  |
| } |  |  |  |

#### -	ReportConfigEUTRA-A3

Table 4.6.6-6: ReportConfigEUTRA-A3

| Derivation Path: 36.331 clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigEUTRA-A3 ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA3 SEQUENCE { |  |  |  |
| a3-Offset | 0 (0 dB) | To reduce interference between intra-frequency multiple cells |  |
| reportOnLeave | FALSE |  |  |
| } |  |  |  |
| } |  |  |  |
| hysteresis | 0 (0 dB) |  |  |
| timeToTrigger | ms640 |  |  |
| } |  |  |  |
| } |  |  |  |
| triggerQuantity | rsrp |  |  |
| reportQuantity | both |  |  |
| maxReportCells | 1 |  |  |
| reportInterval | ms1024 |  |  |
| reportAmount | r1 |  |  |
| si-RequestForHO-r9 | Not Present |  |  |
| ue-RxTxTimeDiffPeriodical-r9 | Not Present |  |  |
| includeLocationInfo-r10 | Not Present |  |  |
| reportAddNeighMeas-r10 | Not Present |  |  |
| alternativeTimeToTrigger-r12 | Not Present |  |  |
| useT312-r12 | Not Present |  |  |
| usePSCell-r12 | Not Present |  |  |
| aN-Threshold1-v1250 | Not Present |  |  |
| a5-Threshold2-v1250 | Not Present |  |  |
| reportStrongestCSI-RSs-r12 | Not Present |  |  |
| reportCRS-Meas-r12 | Not Present |  |  |
| triggerQuantityCSI-RS-r12 | Not Present |  |  |
| reportSSTD-Meas-r13 | Not Present |  |  |
| rs-sinr-Config-r13 | Not Present |  |  |
| useWhiteCellList-r13 | Not Present |  |  |
| measRSSI-ReportConfig-r13 | Not Present |  |  |
| includeMultiBandInfo-r13 | Not Present |  |  |
| ul-DelayConfig-r13 | Not Present |  |  |
| ue-RxTxTimeDiffPeriodicalTDD-r13 | Not Present |  |  |
| purpose-v1430 | Not Present |  |  |
| maxReportRS-Index-r15 | Not Present |  |  |
| includeBT-Meas-r15 | Not Present |  |  |
| includeWLAN-Meas-r15 | Not Present |  |  |
| purpose-r15 | Not Present |  |  |
| numberOfTriggeringCells-r15 | 2 |  | Aerial UE |
| a4-a5-ReportOnLeave-r15 | Not Present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Aerial UE | For Aerial vehicles |

#### -	ReportConfigEUTRA-A4

Table 4.6.6-6AA: ReportConfigEUTRA-A4(Thres)

| Derivation Path: 36.331 clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigEUTRA-A4 ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA4 SEQUENCE { |  |  |  |
| a4-Threshold CHOICE{ |  |  |  |
| threshold-RSRP | Thres + 140 | Thres is actual threshold value in dBm | Not RSRQ |
| threshold-RSRQ | Thres * 2 + 40 | Thres is actual threshold value in dB | RSRQ |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| hysteresis | 0 (0 dB) |  |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| triggerQuantity | rsrp |  | Not RSRQ |
|  | rsrq |  | RSRQ |
| reportQuantity | both |  |  |
| maxReportCells | 1 |  |  |
| reportInterval | ms1024 |  |  |
| reportAmount | r1 |  |  |
| si-RequestForHO-r9 | Not Present |  |  |
| ue-RxTxTimeDiffPeriodical-r9 | Not Present |  |  |
| includeLocationInfo-r10 | Not Present |  |  |
| reportAddNeighMeas-r10 | Not Present |  |  |
| alternativeTimeToTrigger-r12 | Not Present |  |  |
| useT312-r12 | Not Present |  |  |
| usePSCell-r12 | Not Present |  |  |
| aN-Threshold1-v1250 | Not Present |  |  |
| a5-Threshold2-v1250 | Not Present |  |  |
| reportStrongestCSI-RSs-r12 | Not Present |  |  |
| reportCRS-Meas-r12 | Not Present |  |  |
| triggerQuantityCSI-RS-r12 | Not Present |  |  |
| reportSSTD-Meas-r13 | Not Present |  |  |
| rs-sinr-Config-r13 | Not Present |  |  |
| useWhiteCellList-r13 | Not Present |  |  |
| measRSSI-ReportConfig-r13 | Not Present |  |  |
| includeMultiBandInfo-r13 | Not Present |  |  |
| ul-DelayConfig-r13 | Not Present |  |  |
| ue-RxTxTimeDiffPeriodicalTDD-r13 | Not Present |  |  |
| purpose-v1430 | Not Present |  |  |
| maxReportRS-Index-r15 | Not Present |  |  |
| includeBT-Meas-r15 | Not Present |  |  |
| includeWLAN-Meas-r15 | Not Present |  |  |
| purpose-r15 | Not Present |  |  |
| numberOfTriggeringCells-r15 | 2 |  | Aerial UE |
| a4-a5-ReportOnLeave-r15 | Not Present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| RSRQ | When RSRQ based measurement is applied |
| Aerial UE | See the definition below table 4.6.6-6. |

#### -	ReportConfigEUTRA-A5

Table 4.6.6-6AB: ReportConfigEUTRA-A5(Thres1, Thres2)

| Derivation Path: 36.331 clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigEUTRA-A5 ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA5 SEQUENCE { |  |  |  |
| a5-Threshold1 CHOICE{ |  |  |  |
| threshold-RSRP | Thres1 + 140 | Thres1 is actual threshold value in dBm | Not RSRQ |
| threshold-RSRQ | Thres1 * 2 + 40 | Thres1 is actual threshold value in dB | RSRQ |
| } |  |  |  |
| a5-Threshold2 CHOICE{ |  |  |  |
| threshold-RSRP | Thres2 + 140 | Thres2 is actual threshold value in dBm | Not RSRQ |
| threshold-RSRQ | Thres2 * 2 + 40 | Thres2 is actual threshold value in dB | RSRQ |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| hysteresis | 0 (0 dB) |  |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| triggerQuantity | rsrp |  | Not RSRQ |
|  | rsrq |  | RSRQ |
| reportQuantity | both |  |  |
| maxReportCells | 1 |  |  |
| reportInterval | ms1024 |  |  |
| reportAmount | r1 |  |  |
| si-RequestForHO-r9 | Not Present |  |  |
| ue-RxTxTimeDiffPeriodical-r9 | Not Present |  |  |
| includeLocationInfo-r10 | Not Present |  |  |
| reportAddNeighMeas-r10 | Not Present |  |  |
| alternativeTimeToTrigger-r12 | Not Present |  |  |
| useT312-r12 | Not Present |  |  |
| usePSCell-r12 | Not Present |  |  |
| aN-Threshold1-v1250 | Not Present |  |  |
| a5-Threshold2-v1250 | Not Present |  |  |
| reportStrongestCSI-RSs-r12 | Not Present |  |  |
| reportCRS-Meas-r12 | Not Present |  |  |
| triggerQuantityCSI-RS-r12 | Not Present |  |  |
| reportSSTD-Meas-r13 | Not Present |  |  |
| rs-sinr-Config-r13 | Not Present |  |  |
| useWhiteCellList-r13 | Not Present |  |  |
| measRSSI-ReportConfig-r13 | Not Present |  |  |
| includeMultiBandInfo-r13 | Not Present |  |  |
| ul-DelayConfig-r13 | Not Present |  |  |
| ue-RxTxTimeDiffPeriodicalTDD-r13 | Not Present |  |  |
| purpose-v1430 | Not Present |  |  |
| maxReportRS-Index-r15 | Not Present |  |  |
| includeBT-Meas-r15 | Not Present |  |  |
| includeWLAN-Meas-r15 | Not Present |  |  |
| purpose-r15 | Not Present |  |  |
| numberOfTriggeringCells-r15 | 2 |  | Aerial UE |
| a4-a5-ReportOnLeave-r15 | Not Present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| RSRQ | See the definition below table 4.6.6-6AA. |
| Aerial UE | See the definition below table 4.6.6-6. |

#### -	ReportConfigEUTRA-A6

Table 4.6.6-6A: ReportConfigEUTRA-A6

| Derivation Path: 36.331 clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigEUTRA-A6 ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA6-r10 SEQUENCE { |  |  |  |
| a6-Offset-r10 | 0 (0 dB) |  |  |
| a6-ReportOnLeave-r10 | FALSE |  |  |
| } |  |  |  |
| } |  |  |  |
| hysteresis | 0 (0 dB) |  |  |
| timeToTrigger | ms640 |  |  |
| } |  |  |  |
| } |  |  |  |
| triggerQuantity | rsrp |  |  |
| reportQuantity | both |  |  |
| maxReportCells | 1 |  |  |
| reportInterval | ms1024 |  |  |
| reportAmount | r1 |  |  |
| si-RequestForHO-r9 | Not Present |  |  |
| ue-RxTxTimeDiffPeriodical-r9 | Not Present |  |  |
| includeLocationInfo-r10 | Not Present |  |  |
| reportAddNeighMeas-r10 | Not Present |  |  |
| } |  |  |  |

#### -	ReportConfigEUTRA-PERIODICAL

Table 4.6.6-7: ReportConfigEUTRA-PERIODICAL

| Derivation Path: 36.331 clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigEUTRA-PERIODICAL ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| purpose | reportStrongestCells |  |  |
| } |  |  |  |
| } |  |  |  |
| triggerQuantity | rsrp |  |  |
| reportQuantity | both |  |  |
| maxReportCells | 1 |  |  |
| reportInterval | ms1024 |  |  |
| reportAmount | infinity |  |  |
| } |  |  |  |

#### -	ReportConfigInterRAT-B1-GERAN

Table 4.6.6-7A: ReportConfigInterRAT-B1-GERAN(GERAN-Thres)

| Derivation Path: 36.331 clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigInterRAT-B1-GERAN(GERAN-Thres) ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventB1 SEQUENCE { |  |  |  |
| b1-Threshold CHOICE { |  |  |  |
| b1-ThresholdGERAN | (GERAN-Thres + 110) | GERAN-Thres is actual value in dBm |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| hysteresis | 0 | INTEGER(0..30) |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| maxReportCells | 6 | In line with RAN4 requirement |  |
| reportInterval | ms1024 |  |  |
| reportAmount | r1 |  |  |
| } |  |  |  |

#### -	ReportConfig-B1-NR-r15

Table 4.6.6-7AA: ReportConfig-B1-NR-r15(NR_Thres)

| Derivation Path: 36.331 clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfig-B1-NR ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventB1-NR-r15 SEQUENCE{ |  |  |  |
| b1-ThresholdNR-r15 CHOICE{ |  |  |  |
| nr-RSRP-r15 | NR_Thres | NR_Thres is actual RSCP value in dBm |  |
| } |  |  |  |
| reportOnLeave-r15 | FALSE |  |  |
| } |  |  |  |
| } |  |  |  |
| hysteresis | 0 |  |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| maxReportCells | 6 |  |  |
| reportInterval | ms1024 |  |  |
| reportAmount | r1 |  |  |
| } |  |  |  |

#### -	ReportConfigInterRAT-B1-UTRA

Table 4.6.6-7B: ReportConfigInterRAT-B1-UTRA(UTRA-Thres)

| Derivation Path: 36.331 clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigInterRAT-B1-UTRA(UTRA-Thres) ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventB1 SEQUENCE { |  |  |  |
| b1-Threshold CHOICE { |  |  |  |
| b1-ThresholdUTRA CHOICE { |  |  |  |
| utra-EcN0 | (UTRA-Thres * 2 + 49) | UTRA-Thres is actual Ec/NO value in dB | UTRA-FDD |
| utra-RSCP | UTRA-Thres + 115 | For signalling test cases UTRA-Thres is actual RSCP value in dBm | UTRA-FDD |
| utra-RSCP | UTRA-Thres + 115 | UTRA-Thres is actual RSCP value in dBm | UTRA-TDD |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| hysteresis | 3 (1.5 dB) |  |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| maxReportCells | 6 | In line with RAN4 requirement |  |
| reportInterval | ms1024 |  |  |
| reportAmount | r1 |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

#### -	ReportConfigInterRAT-B2-CDMA2000

Table 4.6.6-7C: ReportConfigInterRAT-B2-CDMA2000(EUTRA-Thres, CDMA2000-Thres)

| Derivation Path: 36.331 clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigInterRAT-B2-CDMA2000(EUTRA-Thres, CDMA2000-Thres) ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventB2 SEQUENCE { |  |  |  |
| b2-Threshold1 CHOICE { |  |  |  |
| threshold-RSRP | EUTRA-Thres+140 | EUTRA-Thres is actual threshold value in dBm |  |
| } |  |  |  |
| b2-Threshold2 CHOICE { |  |  |  |
| b2-Threshold2CDMA2000 | [30 (-15 dB)] | Integer (0..63) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| Hysteresis | [2 (1 dB)] | INTEGER(0..30) |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| maxReportCells | [8] |  |  |
| reportInterval | [ms2048] | Range: ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, min1, min6, min12, min30, min60, spare3, spare2, spare1 |  |
| reportAmount | [r1] | Range: r1, r2, r4, r8, r16, r32, r64, infinity |  |
| } |  |  |  |

Table 4.6.6-7D: Void

#### -	ReportConfigInterRAT-B2-GERAN

Table 4.6.6-7E: ReportConfigInterRAT-B2-GERAN(EUTRA-Thres, GERAN-Thres)

| Derivation Path: 36.331 clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigInterRAT-B2-GERAN(EUTRA-Thres, GERAN-Thres) ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventB2 SEQUENCE { |  |  |  |
| b2-Threshold1 CHOICE { |  |  |  |
| threshold-RSRP | EUTRA-Thres+140 | EUTRA-Thres is actual threshold value in dBm |  |
| } |  |  |  |
| b2-Threshold2 CHOICE { |  |  |  |
| b2-Threshold2GERAN | GERAN-Thres + 110 | GERAN-Thres is actual value in dBm |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| hysteresis | 0 | INTEGER(0..30) |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| maxReportCells | 6 | In line with RAN4 requirement |  |
| reportInterval | ms1024 |  |  |
| reportAmount | r1 |  |  |
| } |  |  |  |

#### -	ReportConfigInterRAT-B2-UTRA

Table 4.6.6-8: ReportConfigInterRAT-B2-UTRA(EUTRA-Thres, UTRA-Thres)

| Derivation Path: 36.331 clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigInterRAT-B2-UTRA(EUTRA-Thres, UTRA-Thres) ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventB2 SEQUENCE { |  |  |  |
| b2-Threshold1 CHOICE { |  |  |  |
| threshold-RSRP | EUTRA-Thres+140 | EUTRA-Thres is actual threshold value in dBm |  |
| } |  |  |  |
| b2-Threshold2 CHOICE { |  |  |  |
| b2-Threshold2UTRA CHOICE { |  |  |  |
| utra-EcN0 | UTRA-Thres * 2 + 49 | UTRA-Thres is actual Ec/NO value in dB | UTRA-FDD |
| utra-RSCP | UTRA-Thres + 115 | For signalling test cases UTRA-Thres is actual RSCP value in dBm | UTRA-FDD |
| utra-RSCP | UTRA-Thres + 115 | UTRA-Thres is actual RSCP value in dBm | UTRA-TDD |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| hysteresis | 3 (1.5 dB) |  |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| maxReportCells | 6 | In line with RAN4 requirement |  |
| reportInterval | ms1024 |  |  |
| reportAmount | r1 |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

#### -	ReportConfigInterRAT-B2-NR

Table 4.6.6-8A: ReportConfigInterRAT-B2-NR(EUTRA-Thres, NR-Thres)

| Derivation Path: 36.331 clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigInterRAT-B2-NR ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventB2-NR-r15 SEQUENCE{ |  |  |  |
| b2-Threshold1-r15 CHOICE{ |  |  |  |
| threshold-RSRP | EUTRA Thres | EUTRA Thres is an entry value into a mapping table in TS 36.133 [39] |  |
| } |  |  |  |
| b2-Threshold2NR-r15 CHOICE{ |  |  |  |
| nr-RSRP-r15 | NR Thres | NR Thres is an entry value into a mapping table in TS 38.133 [72] |  |
| } |  |  |  |
| reportOnLeave-r15 | FALSE |  |  |
| } |  |  |  |
| } |  |  |  |
| Hysteresis | 3 (1.5 dB) |  |  |
| timeToTrigger | ms1024 |  |  |
| } |  |  |  |
| } |  |  |  |
| reportInterval | ms1024 |  |  |
| reportAmount | infinity |  |  |
| reportQuantityCellNR-r15 SEQUENCE { |  |  |  |
| ss-rsrp | TRUE |  |  |
| ss-rsrq | FALSE |  |  |
| ss-sinr | FALSE |  |  |
| } |  |  |  |
| maxReportRS-Index-r15 | Not present |  |  |
| reportQuantityRS-IndexNR-r15 | Not present |  |  |
| reportRS-IndexResultsNR | Not present |  |  |
| reportSFTD-Meas-r15 | Not present |  |  |
| } |  |  |  |

#### -	-	ReportConfigInterRAT-SFTD

Table 4.6.6-8B: ReportConfigInterRAT-SFTD

| Derivation Path: 36.331 clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigInterRAT-PERIODICAL ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| purpose | reportStrongestCells |  |  |
| } |  |  |  |
| } |  |  |  |
| maxReportCells | 1 |  |  |
| reportInterval | ms1024 |  |  |
| reportAmount | r1 |  |  |
| si-RequestForHO-r9 | Not present |  |  |
| reportQuantityUTRA-FDD-r10 | Not present |  |  |
| includeLocationInfo-r11 | Not present |  |  |
| b2-Threshold1-v1250 | Not present |  |  |
| reportQuantityWLAN-r13 | Not present |  |  |
| reportQuantityCellNR-r15 | Not present |  |  |
| maxReportRS-Index-r15 |  |  |  |
| reportQuantityRS-IndexNR-r15 |  |  |  |
| reportRS-IndexResultsNR |  |  |  |
| reportSFTD-Meas-r15 | neighborCells |  | INTER-RAT |
|  | pSCell |  | INTRA-FREQ |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| INTER-RAT | For Inter-RAT SFTD measurement |
| INTRA-FREQ | For Intra-frequency SFTD measurement |

#### ReportConfigInterRAT-PERIODICAL

Table 4.6.6-9: ReportConfigInterRAT-PERIODICAL

| Derivation Path: 36.331 clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigInterRAT-PERIODICAL ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| purpose | reportStrongestCells |  |  |
| } |  |  |  |
| } |  |  |  |
| maxReportCells | 1 |  |  |
| reportInterval | ms1024 |  |  |
| reportAmount | infinity |  |  |
| } |  |  |  |

Table 4.6.6-9A: ReportConfigInterRAT-W1-WLAN (ThreshDB)

| Derivation Path: 36.331 clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ReportConfigInterRAT ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { | 1 entry |  |  |
| eventID CHOICE { |  |  |  |
| eventW1-r13 SEQUENCE { | 1 entry |  |  |
| w1-Threshold-r13 | ThreshDB+101 |  |  |
| } |  |  |  |
| hysteresis | 2 | 2 * .5 is 1dB |  |
| timeToTrigger | ms5120 | 5120ms |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	ReportConfigInterRAT-W2-WLAN

Table 4.6.6-9B: ReportConfigInterRAT-W2-WLAN (Thresh1DB, Thresh2DB)

| Derivation Path: 36.331 clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ReportConfigInterRAT::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { | 1 entry |  |  |
| eventID CHOICE { |  |  |  |
| eventW2-r13 SEQUENCE { | 2 entries |  |  |
| w2-Threshold1-r13 | Thresh1DB+101 |  |  |
| w2-Threshold2-r13 | Thresh2DB+101 |  |  |
| } |  |  |  |
| hysteresis | 2 | 2 * .5 is 1dB |  |
| timeToTrigger | ms5120 | 5120ms |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	ReportConfigInterRAT-W3-WLAN

Table 4.6.6-9C: ReportConfigInterRAT-W3-WLAN (ThreshDB)

| Derivation Path: 36.331 clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ReportConfigInterRAT::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { | 1 entry |  |  |
| eventID CHOICE { |  |  |  |
| eventW3-r13 SEQUENCE { | 1 entry |  |  |
| W3-Threshold-r13 | ThreshDB+101 |  |  |
| } |  |  |  |
| hysteresis | 2 | 2 * .5 is 1dB |  |
| timeToTrigger | ms5120 | 5120ms |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### -	ReportConfigEUTRA-C1

Table 4.6.6-10: ReportConfigEUTRA-C1(Thres)

| Derivation Path: 36.331 clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigEUTRA-C1 ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventC1-r12 SEQUENCE { |  |  |  |
| c1-Threshold-r12 | Thres+140 | Thres is actual threshold value in dBm |  |
| c1-ReportOnLeave-r12 | FALSE |  |  |
| } |  |  |  |
| } |  |  |  |
| hysteresis | 0 (0 dB) |  |  |
| timeToTrigger | ms640 |  |  |
| } |  |  |  |
| } |  |  |  |
| triggerQuantity | rsrp |  |  |
| reportQuantity | sameAsTriggerQuantity |  |  |
| maxReportCells | 1 |  |  |
| reportInterval | ms1024 |  |  |
| reportAmount | r1 |  |  |
| si-RequestForHO-r9 | Not Present |  |  |
| ue-RxTxTimeDiffPeriodical-r9 | Not Present |  |  |
| includeLocationInfo-r10 | Not Present |  |  |
| reportAddNeighMeas-r10 | Not Present |  |  |
| alternativeTimeToTrigger-r12 | Not Present |  |  |
| useT312-r12 | Not Present |  |  |
| usePSCell-r12 | Not Present |  |  |
| aN-Threshold1-v1250 | Not Present |  |  |
| a5-Threshold2-v1250 | Not Present |  |  |
| reportStrongestCSI-RSs-r12 | false |  |  |
| reportCRS-Meas-r12 | false |  |  |
| triggerQuantityCSI-RS-r12 | true |  |  |
| } |  |  |  |

#### -	ReportConfigEUTRA-C2

Table 4.6.6-11: ReportConfigEUTRA-C2

| Derivation Path: 36.331 clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigEUTRA-C2 ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventC2-r12 SEQUENCE { |  |  |  |
| c2-RefCSI-RS-r12 | 1 |  |  |
| c2-Offset-r12 | 2 | The actual value is IE value * 0.5 dB. |  |
| c2-ReportOnLeave-r12 | FALSE |  |  |
| } |  |  |  |
| } |  |  |  |
| hysteresis | 0 (0 dB) |  |  |
| timeToTrigger | ms640 |  |  |
| } |  |  |  |
| } |  |  |  |
| triggerQuantity | rsrp |  |  |
| reportQuantity | sameAsTriggerQuantity |  |  |
| maxReportCells | 1 |  |  |
| reportInterval | ms1024 |  |  |
| reportAmount | r1 |  |  |
| si-RequestForHO-r9 | Not Present |  |  |
| ue-RxTxTimeDiffPeriodical-r9 | Not Present |  |  |
| includeLocationInfo-r10 | Not Present |  |  |
| reportAddNeighMeas-r10 | Not Present |  |  |
| alternativeTimeToTrigger-r12 | Not Present |  |  |
| useT312-r12 | Not Present |  |  |
| usePSCell-r12 | Not Present |  |  |
| aN-Threshold1-v1250 | Not Present |  |  |
| a5-Threshold2-v1250 | Not Present |  |  |
| reportStrongestCSI-RSs-r12 | false |  |  |
| reportCRS-Meas-r12 | false |  |  |
| triggerQuantityCSI-RS-r12 | true |  |  |
| } |  |  |  |

#### -	ReportConfigEUTRA-PERIODICAL-CSI-RS

Table 4.6.6-12: ReportConfigEUTRA-PERIODICAL-CSI-RS

| Derivation Path: 36.331 clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigEUTRA-PERIODICAL ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| purpose | reportStrongestCells |  |  |
| } |  |  |  |
| } |  |  |  |
| triggerQuantity | rsrp |  |  |
| reportQuantity | both |  |  |
| maxReportCells | 1 |  |  |
| reportInterval | ms1024 |  |  |
| reportAmount | infinity |  |  |
| si-RequestForHO-r9 | Not Present |  |  |
| ue-RxTxTimeDiffPeriodical-r9 | Not Present |  |  |
| includeLocationInfo-r10 | Not Present |  |  |
| reportAddNeighMeas-r10 | Not Present |  |  |
| alternativeTimeToTrigger-r12 | Not Present |  |  |
| useT312-r12 | Not Present |  |  |
| usePSCell-r12 | Not Present |  |  |
| aN-Threshold1-v1250 | Not Present |  |  |
| a5-Threshold2-v1250 | Not Present |  |  |
| reportStrongestCSI-RSs-r12 | true |  |  |
| reportCRS-Meas-r12 | false |  |  |
| triggerQuantityCSI-RS-r12 | false |  |  |
| } |  |  |  |

#### -	ReportConfigEUTRA-V1

Table 4.6.6-13: ReportConfigEUTRA-V1

| Derivation Path: 36.331 clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigEUTRA-V1 ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventV1-r14 SEQUENCE { |  |  |  |
| v1-Threshold-r14 | 50 |  |  |
| } |  |  |  |
| } |  |  |  |
| hysteresis | 0 (0 dB) |  |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| triggerQuantity | rsrp |  | Not RSRQ |
| reportQuantity | both |  |  |
| maxReportCells | 1 |  |  |
| reportInterval | ms1024 |  |  |
| reportAmount | r1 |  |  |
| purpose-v14xy | sidelink |  |  |
| } |  |  |  |

#### -	ReportConfigEUTRA-V2

Table 4.6.6-14: ReportConfigEUTRA-V2

| Derivation Path: 36.331 clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigEUTRA-V2 ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventV2-r14 SEQUENCE { |  |  |  |
| v2-Threshold-r14 | 10 |  |  |
| } |  |  |  |
| } |  |  |  |
| hysteresis | 0 (0 dB) |  |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| triggerQuantity | rsrp |  | Not RSRQ |
| reportQuantity | both |  |  |
| maxReportCells | 1 |  |  |
| reportInterval | ms1024 |  |  |
| reportAmount | r1 |  |  |
| purpose-v14xy | sidelink |  |  |
| } |  |  |  |

#### -	ReportConfigEUTRA-H1

Table 4.6.6-15: ReportConfigEUTRA-H1

| Derivation Path: 36.331 clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigEUTRA-H1 ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventH1-r15 SEQUENCE { |  |  |  |
| h1-ThresholdOffset-r15 | 0 (0m) |  |  |
| h1-Hysteresis-15 | 5 (5m) |  |  |
| } |  |  |  |
| } |  |  |  |
| hysteresis | 0 (0 dB) | IE ignored by UE when h1-Hysteresis-15 is configured. |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| triggerQuantity | rsrp | IE ignored by UE for event H1 |  |
| reportQuantity | both | IE ignored by UE for event H1 |  |
| maxReportCells | 1 | IE ignored by UE for event H1 |  |
| reportInterval | ms1024 |  |  |
| reportAmount | r1 |  |  |
| } |  |  |  |

#### -	ReportConfigEUTRA-H2

Table 4.6.6-16: ReportConfigEUTRA-H2

| Derivation Path: 36.331 clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigEUTRA-H2 ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventH2-r15 SEQUENCE { |  |  |  |
| h2-ThresholdOffset-r15 | 15 (30m) |  |  |
| h2-Hysteresis-15 | 10 (10m) |  |  |
| } |  |  |  |
| } |  |  |  |
| hysteresis | 0 (0 dB) | IE ignored by UE when h2-Hysteresis-15 is configured. |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| triggerQuantity | rsrp | IE ignored by UE for event H2 |  |
| reportQuantity | both | IE ignored by UE for event H2 |  |
| maxReportCells | 1 | IE ignored by UE for event H2 |  |
| reportInterval | ms1024 |  |  |
| reportAmount | r1 |  |  |
| } |  |  |  |

### 4.6.7	Other information elements

#### –	FlightPathInfoReportConfig

Table 4.6.7-0: FlightPathInfoReportConfig

| Derivation Path: 36.331 clause 6.3.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| FlightPathInfoReportConfig-r15 ::= SEQUENCE { |  |  |  |  |
| maxWayPointNumber-r15 |  | 1 |  |  |
| includeTimeStamp-r15 |  | Not present |  |  |
| } |  |  |  |  |

#### –	RRC-TransactionIdentifier-DL

Table 4.6.7-1: RRC-TransactionIdentifier-DL

| Derivation Path: 36.331 clause 6.3.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRC-TransactionIdentifier-DL ::= |  | 0..3 |  |  |

#### –	RRC-TransactionIdentifier-UL

Table 4.6.7-2: RRC-TransactionIdentifier-UL

| Derivation Path: 36.331 clause 6.3.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRC-TransactionIdentifier-UL ::= |  | 0..3 | The same value as the value of RRC-TransactionIdentifier-DL in the downlink message initiating the procedure |  |

### 4.6.8	Channel-bandwidth-dependent parameters

The default values of parameters which depend on the channel bandwidth are defined in table 4.6.8-1.

Table 4.6.8-1: Channel-bandwidth-dependent parameters

| Information Element | Channel bandwidth |  |  |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20MHz |  |
| Prach-FrequencyOffset or prach-FreqOffsetHighSpeed-r14 | 0 | 1 | 2 | 4 | 6 | 8 | Typical value in real network |
| nRB-CQI | 0 | 0 | 2 | 4 | 6 | 8 | Selected based on typical maximum number of UEs. |
| Pusch-HoppingOffset | 0 | 2 | 4 | 8 | 12 | 16 | Typical value in real network |
| sr-PUCCH-ResourceIndex | 0 | 10 | 20 | 41 | 62 | 84 |  |
| sr-SlotSPUCCH-IndexFH-r15 | 0 | 10 | 20 | 41 | 62 | 84 |  |
| sr-SlotSPUCCH-IndexNoFH-r15 | 0 | 10 | 20 | 41 | 62 | 84 |  |
| sr-SubslotSPUCCH-ResourceList -r15 | 0 | 10 | 20 | 41 | 62 | 84 |  |
| srs-BandwidthConfig | FFS | FFS | bw3 (mSRS,b, Nb) = (20, 1), (4, 5), (4, 1), (4, 1) | bw2 (mSRS,b, Nb) = (40, 1), (20, 2), (4, 5), (4, 1) | bw2 (mSRS,b, Nb) = (60, 1), (20, 3), (4, 5), (4, 1) | bw2 (mSRS,b, Nb) = (80, 1), (40, 2), (20, 2), (4, 5) | Selected in accordance with pucch-ResourceSize. |

## 4.7	Default NAS message and information element contents

This clause contains the default values of common NAS messages and  information elements, which apply to all test cases unless otherwise specified. All the messages and information elements are listed in alphabetical order.

### 4.7.1	Security protected NAS messages

In subclauses 4.7.2 and 4.7.3 in this document, all the NAS messages are described in the plain NAS message format.

When a NAS message is security protected, the message shall be contained by SECURITY PROTECTED NAS MESSAGE unless contained by another NAS message.

The default contents of SECURITY PROTECTED NAS MESSAGE message are defined in table 4.7.1-1.

Table 4.7.1-1: SECURITY PROTECTED NAS MESSAGE

| Derivation Path: 24.301 clause 8.2.23 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | EMM |  |  |
| Security header type |  | '0001'B | Integrity protected | UNCIPHERED |
|  |  | '0010'B | Integrity protected and ciphered | CIPHERED |
|  |  | '0011'B | Integrity protected with new EPS security context | UNCIPHERED-NEW |
|  |  | '0100'B | Integrity protected and ciphered with new EPS security context | CIPHERED-NEW |
|  |  | '0101'B | Integrity protected and partially ciphered NAS message | CIPHERED-PARTIALLY |
| Message authentication code |  | The calculated value of MAC-I for this message. | The value of MAC-I is calculated by SS using Sequence number sent by UE. | SENT-BY-SS |
|  |  | The same value as the XMAC-I value calculated by SS. |  | SENT-BY-UE |
| Sequence number |  | The internal counter of the SS |  | SENT-BY-SS |
|  |  | Any allowed value |  | SENT-BY-UE |
| NAS message |  | Set according to specific message content |  |  |

| Condition | Explanation |
| --- | --- |
| UNCIPHERED | This condition applies to unciphered NAS message exchange |
| CIPHERED | This condition applies to ciphered NAS message exchange |
| UNCIPHERED-NEW | This condition applies to unciphered NAS message exchange with new EPS security context |
| CIPHERED-NEW | This condition applies to ciphered NAS message exchange with new EPS security context |
| CIPHERED-PARTIALLY | This condition applies to partially ciphered NAS message exchange |
| SENT-BY-SS | Use for the message sent from SS to UE |
| SENT-BY-UE | Use for the message sent from UE to SS |

When a valid NAS security context exists, unless specified otherwise in other clauses of the present specification or in a test case, for every DL NAS message sent within SECURITY PROTECTED NAS MESSAGE message, the condition CIPHERED applies.

### 4.7.2	Contents of EMM messages

#### -	ATTACH ACCEPT

This message is sent by the SS to the UE.

Table 4.7.2-1: ATTACH ACCEPT

| Derivation Path: 24.301 clause 8.2.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | EMM |  |  |
| Security header type |  | '0000'B | Plain NAS message, not security protected |  |
| Attach accept message identity |  | '0100 0010'B | Attach accept |  |
| EPS attach result |  | '0001'B | EPS only | EPS_only |
|  |  | '0010'B | combined EPS/IMSI attach | combined_EPS_IMSI |
| Spare half octet |  | '0000'B |  |  |
| T3412 value |  |  |  |  |
| Timer value |  | '0 0000'B |  |  |
| Unit |  | '111'B | value indicates that the timer is deactivated. |  |
| TAI list |  |  |  |  |
| Length of tracking area identity list contents |  | '0000 0110'B | 6 octets |  |
| Partial tracking area identity list 1 |  |  |  |  |
| Number of elements |  | '0 0000'B | 1 element |  |
| Type of list |  | '00'B | list of TACs belonging to one PLMN, with non-consecutive TAC values |  |
| MCC |  | See table 4.4.2-2 in this document | For NAS test cases, see table 6.3.2.2-1. |  |
| MNC |  | See table 4.4.2-2 in this document | For NAS test cases, see table 6.3.2.2-1. |  |
| TAC 1 |  | See table 4.4.2-2 in this document | For NAS test cases, see table 6.3.2.2-1. |  |
| ESM message container |  | ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message to activate the default bearer |  |  |
| ESM message container |  | ESM DUMMY MESSAGE |  | CIoT_Attach_WithoutPDN |
| GUTI |  |  |  |  |
| Length of EPS mobile identity contents |  | '0000 1011'B | 11 octets |  |
| Type of identity |  | '110'B | GUTI |  |
| Odd/even indication |  | '0'B | even number of identity digits and also when the GUTI is used |  |
| MCC |  | See table 4.4.2-2 in this document | For NAS test cases, see table 6.3.2.2-1. |  |
| MNC |  | See table 4.4.2-2 in this document | For NAS test cases, see table 6.3.2.2-1. |  |
| MME Group ID |  | See table 4.4.2-2 in this document | For NAS test cases, see table 6.3.2.2-1. |  |
| MME Code |  | See table 4.4.2-2 in this document | For NAS test cases, see table 6.3.2.2-1. |  |
| M-TMSI |  | See table 4.4.2-2 in this document | For NAS test cases, see table 6.3.2.2-1. |  |
| Location area identification |  | Not present |  | EPS_only |
| Location area identification |  |  |  | combined_EPS_IMSI |
| MCC |  | MCC of the EUTRA cell from which this message is sent |  |  |
| MNC |  | MCC of the EUTRA cell from which this message is sent |  |  |
| LAC |  | 1 |  |  |
| MS identity |  | Not present |  | EPS_only |
| MS identity |  |  |  | combined_EPS_IMSI |
| Length of mobile identity contents |  | '0000 0101'B | 5 octets |  |
| Type of identity |  | '100'B | TMSI/P-TMSI/M-TMSI |  |
| Odd/even indication |  | '0'B | even number of identity digits and also when the TMSI/P-TMSI or TMGI and optional MBMS Session Identity is used |  |
| TMSI |  | TMSI-1 |  |  |
| EMM cause |  | Not present |  |  |
| T3402 value |  | Not present |  |  |
| T3423 value |  | Not present |  |  |
| Equivalent PLMNs |  | Not present |  |  |
| Emergency number list |  | Not present |  |  |
| EPS network feature support |  | '0000 0011'B | IMS voice over PS session in S1 mode supported, emergency bearer services in S1 mode supported |  |
| EPS network feature support |  | '0000 0001'B | IMS voice over PS session in S1 mode supported | Rel-8 |
| EPS network feature support |  | ‘1000 0011 0000 1111’B | IMS voice over PS session in S1 mode, emergency bearer services in S1 mode, CP CIoT, ePCO, HC-CP CIoT, S1-U data and UP CIoT supported | CIoT |
| EPS network feature support |  | ‘1100 0000 0000 1100’B | CIoT control plane optimization indicated, Attach Without PDN supported, ePCO supported, Header Compression for CIoT control plane supported | CIoT_CP AND pc_NB |
| EPS network feature support |  | ‘1000 0000 0000 1011’B | CP CIoT supported, ePCO supported, S1-U data transfer supported, CIoT user plane optimization indicated | CIoT_UP AND pc_NB |
| EPS network feature support |  | ‘1000 0000 0000 1010’B | CP CIoT supported, ePCO supported, S1-U data transfer only indicated | S1-U Only AND pc_NB |
| EPS network feature support |  | ‘0010 0000 0000 0000’B | EXTENDED SERVICE REQUEST for packet services indicated | ESRPS |
| EPS network feature support |  | '0000 0011 0000 1000'B | - IMS voice over PS session in S1 mode supported - emergency bearer services in S1 mode supported - ePCO supported - Interworking without N26 interface not supported | NR |
| Additional update result |  | Not present |  |  |
| Additional update result |  | ‘0010’ B | “SMS only” | additional_update_result_SMS OR CIoT_SMSOnly |
| T3412 extended value |  | Not present |  | Rel-10 |
| T3324 value |  | Not present |  |  |
| Extended DRX parameters |  | Not present |  |  |
| DCN-ID |  | Not present |  |  |
| SMS services status |  | Not present |  |  |
| Non-3GPP NW provided policies |  | Not present |  |  |
| T3448 value |  | Not present |  |  |
| Network policy |  | Not present |  |  |
| T3447 value |  | Not present |  |  |
| Extended emergency number list |  | Not present |  |  |
| Ciphering key data |  | Not present |  |  |

| Condition | Explanation |
| --- | --- |
| EPS_only | This condition applies if the UE is configured to initiate EPS attach or if explicitly specified. |
| combined_EPS_IMSI | This condition applies if the UE is configured to initiate combined EPS/IMSI attach or if explicitly specified. This condition does not apply for NB-S1. |
| additional_update_result_SMS | If the UE requested “SMS only” in the Additional update type IE and combined_EPS_IMSI condition applies |
| CIoT_Attach_WithoutPDN | If the UE wants to attach without PDN for CIoT optimization |
| CIoT_CP | If test case requires control plane CIoT optimization |
| CIoT_SMSOnly | If SMS Only is indicated in the ATTACH REQ received earlier |
| CIoT_UP | If test case requires user plane CIoT optimization |
| S1-U data Only | If S1-U data transfer Only is indicated |
| CIoT | If in the last ATTACH REQUEST, the UE network capability was included with the “CP CIoT” (octet 8, bit 3) or the “UP CIoT” and “S1-U data” (octet 8, bit 4 and bit 5). Note 1: This condition is only applicable for E-UTRA UEs (TS 36.523-2 A.4.1-1/1 or A.4.1-1/2). |
| ESRPS | If the test case requires the network to indicate support of EXTENDED SERVICE REQUEST for packet services (ESRPS) |
| NR | Used in NR test cases (TS 38.523-1 [71]) |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

#### -	ATTACH COMPLETE

This message is sent by the UE to the SS.

Table 4.7.2-2: ATTACH COMPLETE

| Derivation Path: 24.301 clause 8.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | EMM |  |  |
| Security header type |  | '0000'B | Plain NAS message, not security protected |  |
| Attach complete message identity |  | '0100 0011'B | Attach complete |  |
| ESM message container |  | ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message |  |  |
| ESM message container |  | ESM DUMMY MESSAGE |  | CIoT_Attach_WithoutPDN |

| Condition |  | Explanation |
| --- | --- | --- |
| CIoT_Attach_WithoutPDN | If the UE wants to attach without PDN for CIoT optimization |  |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

#### -	ATTACH REJECT

This message is sent by the SS to the UE.

Table 4.7.2-3: ATTACH REJECT

| Derivation Path: 24.301 clause 8.2.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | EMM |  |  |
| Security header type |  | '0000'B | Plain NAS message, not security protected |  |
| Attach reject message identity |  | '0100 0100'B | Attach reject |  |
| EMM cause |  | Set according to specific message content. |  |  |
| ESM message container |  | Set according to specific message content. |  |  |

NOTE:	If this message includes the EMM cause set to #3, #6, #7, #8, #11, #12, #13, #14, #15, #25 or #35, it is sent within SECURITY PROTECTED NAS MESSAGE message. Otherwise this message is sent without integrity protection.

#### -	ATTACH REQUEST

This message is sent by the UE to the SS.

Table 4.7.2-4: ATTACH REQUEST

| Derivation Path: 24.301 clause 8.2.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | EMM |  |  |
| Security header type |  | '0000'B | Plain NAS message, not security protected |  |
| Attach request message identity |  | '0100 0001'B | Attach request |  |
| EPS attach type |  | '0001'B | EPS attach | EPS_only |
|  |  | '0010'B | combined EPS/IMSI attach | combined_EPS_IMSI |
| NAS key set identifier |  | Any allowed value |  |  |
| EPS mobile identity |  | Any allowed value |  |  |
| UE network capability |  | Any allowed value |  |  |
| UE network capability |  |  |  | EN-DC |
| All octets with the exception of octet 9, bit 5 |  | Any allowed value |  |  |
| Dual connectivity with NR (DCNR) (octet 9, bit 5) |  | 1 | dual connectivity with NR supported |  |
| UE network capability |  |  |  | NR |
| All octets with the exception of octet 8, bit 8 and octet 9, bit 6 |  | Any allowed value |  |  |
| Extended protocol configuration options (ePCO) (octet 8, bit 8) |  | '1' | Extended protocol configuration options IE supported |  |
| N1 mode supported (N1mode) (octet 9, bit 6) |  | 1 | N1 mode supported |  |
| ESM message container |  | PDN CONNECTIVITY REQUEST message to request PDN connectivity to the default PDN |  |  |
| ESM message container |  | ESM DUMMY MESSAGE |  | CIoT_Attach_WithoutPDN |
| Old P-TMSI signature |  | Not present or any allowed value |  |  |
| Additional GUTI |  | Not present or any allowed value |  |  |
| Last visited registered TAI |  | Not present or any allowed value |  |  |
| DRX parameter |  | Not present or any allowed value |  |  |
| MS network capability |  | Not present or any allowed value |  |  |
| Old location area identification |  | Not present or any allowed value |  |  |
| TMSI status |  | Not present or any allowed value |  |  |
| Mobile station classmark 2 |  | Not present or any allowed value |  |  |
| Mobile station classmark 3 |  | Not present or any allowed value |  |  |
| Supported Codecs |  | Not present or any allowed value |  |  |
| Additional update type |  | Not present |  | EPS_only |
| Additional update type |  | Any allowed value |  | EPS_only_with_SMS |
| Additional update type |  | Not present or any allowed value |  | combined_EPS_IMSI or CIoT |
| Voice domain preference and UE's usage setting |  | Not present or any allowed value |  |  |
| Device properties |  | Not present or any allowed value |  |  |
| Old GUTI type |  | Not present or any allowed value |  |  |
| MS network feature support |  | Not present or any allowed value |  |  |
| TMSI based NRI container |  | Not present or any allowed value |  |  |
| T3324 value |  | Not present or any allowed value |  |  |
| T3412 extended value |  | Not present or any allowed value |  |  |
| Extended DRX parameters |  | Not present or any allowed value |  |  |
| UE additional security capability |  | Any allowed value (NOTE 1, NOTE 2) |  | EN-DC OR NR |
| UE status |  | Not present or any allowed value |  |  |
| Additional information requested |  | Not present or any allowed value |  |  |
| NOTE 1: For a UE supporting dual connectivity with NR, if the UE supports one of the encryption algorithms for 5GS in bits 8 to 5 of octet 3, it shall support the same algorithms for E-UTRAN as specified in 3GPP TS 33.401 [41]. NOTE 2: For a UE supporting dual connectivity with NR, if the UE supports one of the integrity protection algorithms for 5GS in bits 8 to 5 of octet 5, it shall support the same algorithms for E-UTRAN as specified in 3GPP TS 33.401 [41]. |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| EPS_only | See the definition below table 4.7.2-1. |
| combined_EPS_IMSI | See the definition below table 4.7.2-1. |
| CIoT_Attach_WithoutPDN | If the UE wants to attach without PDN for CIoT optimization |
| EPS_only_with_SMS | If the UE wants to perform EPS attach with SMS only for CIoT optimization |
| CIoT | If UE supports CIoT Optimisation |
| EN-DC | Used in E-UTRA-NR dual connectivity test cases (TS 38.523-1 [71]) |
| NR | Used in NR test cases (TS 38.523-1 [71]) |

NOTE:	This message is sent integrity protected when a valid security context exists and without integrity protection otherwise.

#### -	AUTHENTICATION FAILURE

This message is sent by the UE to the SS.

Table 4.7.2-5: AUTHENTICATION FAILURE

| Derivation Path: 24.301 clause 8.2.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | EMM |  |  |
| Security header type |  | '0000'B | Plain NAS message, not security protected |  |
| Authentication failure message type |  | '0101 1100'B | Authentication failure |  |
| EMM cause |  | '0001 0100'B | Mac failure |  |
| Authentication failure parameter |  | Not present | See TS 24.301 [28] subclause 8.2.5.2 |  |

NOTE:	The security protection of this message is the same as the previous AUTHENTICATION REQUEST message.

#### -	AUTHENTICATION REJECT

This message is sent by the SS to the UE.

Table 4.7.2-6: AUTHENTICATION REJECT

| Derivation Path: 24.301 clause 8.2.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | EMM |  |  |
| Security header type |  | '0000'B | Plain NAS message, not security protected |  |
| Authentication reject message type |  | '0101 0100'B | Authentication reject |  |

NOTE:	This message is sent without integrity protection.

#### -	AUTHENTICATION REQUEST

This message is sent by the SS to the UE.

Table 4.7.2-7: AUTHENTICATION REQUEST

| Derivation Path: 24.301 clause 8.2.7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | EMM |  |  |
| Security header type |  | '0000'B | Plain NAS message, not security protected |  |
| Authentication request message type |  | '0101 0010'B | Authentication request |  |
| NAS key set identifierASME |  |  |  |  |
| NAS key set identifier |  | An arbitrarily selected value between '000'B and '110'B, different from the valid NAS key set identifier of the UE if such a value exists. |  |  |
| TSC |  | '0'B | native security context (for KSIASME) |  |
| Spare half octet |  | '0000'B |  |  |
| Authentication parameter RAND (EPS challenge) |  | An arbitrarily selected 128 bits value |  |  |
| Authentication parameter AUTN (EPS challenge) |  | See TS 24.301 [28] subclause 9.9.3.2 |  |  |

NOTE:	Within a test execution this message is sent without integrity protection before NAS security mode control procedure has been successfully completed; and sent integrity protected and ciphered within SECURITY PROTECTED NAS MESSAGE message after NAS security mode control procedure has been successfully completed. SS does not maintain information for NAS security mode control procedure after a TC is completed.

#### -	AUTHENTICATION RESPONSE

This message is sent by the UE to the SS.

Table 4.7.2-8: AUTHENTICATION RESPONSE

| Derivation Path: 24.301 clause 8.2.8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | EMM |  |  |
| Security header type |  | '0000'B | Plain NAS message, not security protected |  |
| Authentication response message type |  | '0101 0011'B | Authentication response |  |
| Authentication response parameter |  | See TS 24.301 [28] subclause 9.9.3.4 |  |  |

NOTE:	When sent in response to an AUTHENTICATION REQUEST message which is not integrity protected and not ciphered, the AUTHENTICATION RESPONSE message may be sent integrity protected when a valid security context exists and without integrity protection otherwise.

#### -	CS SERVICE NOTIFICATION

This message is sent by the SS to the UE.

Table 4.7.2-8A: CS SERVICE NOTIFICATION

| Derivation Path: 24.301 clause 8.2.9 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | EMM |  |  |
| Security header type |  | '0000'B | Plain NAS message, not security protected |  |
| CS service notification message identity |  | '01100100'B | CS Service notification |  |
| Paging identity |  | ‘1’B | TMSI |  |
| CLI |  | Not present |  |  |
| SS Code |  | Not present |  |  |
| LCS indicator |  | Not present |  |  |
| LCS client identity |  | Not present |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

#### -	CONTROL PLANE SERVICE REQUEST

This message is sent by the UE to the SS.

Table 4.7.2-28: CONTROL PLANE SERVICE REQUEST

| Derivation Path: 24.301 clause 8.2.33 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | EMM |  |  |
| Security header type |  | '0000'B | Plain NAS message, not security protected |  |
| Control plane service request message identity |  | '0100 1101'B | Control plane service request |  |
| Data service type |  |  |  |  |
| Data service type value |  | Any allowed value |  |  |
| "Active" flag |  | '0'B | No radio bearer establishment requested |  |
| NAS key set identifier |  |  |  |  |
| NAS key set identifier |  | The valid NAS key set identifier of the UE |  |  |
| TSC |  | '0'B | native security context (for KSIASME) |  |
| ESM message container |  | Not present or PDN CONNECTIVITY REQUEST message to request PDN connectivity to the default PDN |  |  |
| NAS message container |  | Not present or set according to specific message content |  |  |
| EPS bearer context status |  | Not present or (octet 3 = '00100000'B and octet 4 = '00000000'B) |  |  |
| Device properties |  | Not present or Any allowed value |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message as an integrity protected message. If the ESM Message Container or NAS message container is present, this element is ciphered - so the whole message is then integrity protected and partially ciphered.

#### -	DETACH ACCEPT (UE originating detach)

This message is sent by the SS to the UE.

Table 4.7.2-9: DETACH ACCEPT

| Derivation Path: 24.301 clause 8.2.10.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | EMM |  |  |
| Security header type |  | '0000'B | Plain NAS message, not security protected |  |
| Detach accept message identity |  | '0100 0110'B | Detach accept |  |

NOTE:	This message is sent using the same security protection as in the previous DETACH REQUEST message received from the UE.

#### -	DETACH ACCEPT (UE terminated detach)

This message is sent by the UE to the SS.

Table 4.7.2-10: DETACH ACCEPT

| Derivation Path: 24.301 clause 8.2.10.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | EMM |  |  |
| Security header type |  | '0000'B | Plain NAS message, not security protected |  |
| Detach accept message identity |  | '0100 0110'B | Detach accept |  |

NOTE:	This message is sent without integrity protection before NAS security mode control procedure has been successfully completed and sent within SECURITY PROTECTED NAS MESSAGE message after NAS security mode control procedure has been successfully completed.

#### -	DETACH REQUEST (UE originating detach)

This message is sent by the UE to the SS.

Table 4.7.2-11: DETACH REQUEST

| Derivation Path: 24.301 clause 8.2.11.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | EMM |  |  |
| Security header type |  | '0000'B | Plain NAS message, not security protected |  |
| Detach request message identity |  | '0100 0101'B | Detach request |  |
| Detach type |  |  |  |  |
| Type of detach |  | '001'B | EPS detach | EPS_only |
|  |  | ‘011’B | combined EPS/IMSI detach | Combined_EPS_IMSI |
| Switch off |  | '1'B | switch off |  |
| NAS key set identifier |  |  |  |  |
| NAS key set identifier |  | The valid NAS key set identifier. |  |  |
| TSC |  | Any Value |  |  |
| GUTI or IMSI |  | If the UE has a valid GUTI, set to the GUTI, otherwise set to the IMSI of the UE. |  |  |

| Condition | Explanation |
| --- | --- |
| EPS_only | See the definition below table 4.7.2-1. |
| Combined_EPS_IMSI | See the definition below table 4.7.2-1. |

NOTE:	This message is sent with integrity protection before SS has started the ciphering and integrity and ciphered protected after SS has started the ciphering.

#### -	DETACH REQUEST (UE terminated detach)

This message is sent by the SS to the UE.

Table 4.7.2-12: DETACH REQUEST

| Derivation Path: 24.301 clause 8.2.11.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | EMM |  |  |
| Security header type |  | '0000'B | Plain NAS message, not security protected |  |
| Detach request message identity |  | '0100 0101'B | Detach request |  |
| Detach type |  | Set according to specific message content. |  |  |
| Spare half octet |  | '0000'B |  |  |
| EMM cause |  | Set according to specific message content. |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

#### -	DOWNLINK NAS TRANSPORT

This message is sent by the SS to the UE.

Table 4.7.2-12A: DOWNLINK NAS TRANSPORT

| Derivation Path: 24.301 clause 8.2.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | EMM |  |  |
| Security header type |  | '0000'B | Plain NAS message, not security protected |  |
| Downlink NAS transport message identity |  | '0110 0010'B | Downlink NAS transport |  |
| NAS message container |  | Set according to specific message content |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

#### -	EMM INFORMATION

This message is sent by the SS to the UE.

Table 4.7.2-13: EMM INFORMATION

| Derivation Path: 24.301 clause 8.2.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | EMM |  |  |
| Security header type |  | '0000'B | Plain NAS message, not security protected |  |
| EMM information message identity |  | '0110 0001'B | EMM information |  |
| Full name for network |  | Set according to specific message content. |  |  |
| Short name for network |  | Set according to specific message content. |  |  |
| Local time zone |  | Set according to specific message content. |  |  |
| Universal time and local time zone |  | Set according to specific message content. |  |  |
| Network daylight saving time |  | Set according to specific message content. |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

#### -	EMM STATUS

This message is sent by the UE or by the SS.

Table 4.7.2-14: EMM STATUS

| Derivation Path: 24.301 clause 8.2.14 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | EMM |  |  |
| Security header type |  | '0000'B | Plain NAS message, not security protected |  |
| EMM status message identity |  | '0110 0000'B | EMM status |  |
| EMM cause |  | Set according to specific message content. |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

#### -	EXTENDED SERVICE REQUEST

This message is sent by the UE to the SS.

Table 4.7.2-14A: EXTENDED SERVICE REQUEST

| Derivation Path: 24.301 clause 8.2.15 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | EMM |  |  |
| Security header type |  | '0000'B | Plain NAS message, not security protected |  |
| Extended service request message identity |  | '0100 1100'B | Extended service request |  |
| Service type |  | '0001'B | mobile terminating CS fallback or 1xCS fallback |  |
| NAS key set identifier |  |  |  |  |
| NAS key set identifier |  | The valid NAS key set identifier. |  |  |
| TSC |  | '0'B | native security context (for KSIASME) |  |
| M-TMSI |  | If the UE has a valid M-TMSI, set to the M-TMSI, otherwise set to the IMSI of the UE. |  |  |
| CSFB response |  | ’001’B | CS fallback accepted by the UE |  |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

#### -	GUTI REALLOCATION COMMAND

This message is sent by the SS to the UE.

Table 4.7.2-15: GUTI REALLOCATION COMMAND

| Derivation Path: 24.301 clause 8.2.16 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | EMM |  |  |
| Security header type |  | '0000'B | Plain NAS message, not security protected |  |
| GUTI reallocation command message identity |  | '0101 0000'B | GUTI reallocation command |  |
| GUTI |  | Set according to specific message content. |  |  |
| TAI list |  | Set according to specific message content. |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

#### -	GUTI REALLOCATION COMPLETE

This message is sent by the UE to the SS.

Table 4.7.2-16: GUTI REALLOCATION COMPLETE

| Derivation Path: 24.301 clause 8.2.17 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | EMM |  |  |
| Security header type |  | '0000'B | Plain NAS message, not security protected |  |
| GUTI reallocation complete message identity |  | '0101 0001'B | GUTI reallocation complete |  |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

#### -	IDENTITY REQUEST

This message is sent by the SS to the UE.

Table 4.7.2-17: IDENTITY REQUEST

| Derivation Path: 24.301 clause 8.2.18 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | EMM |  |  |
| Security header type |  | '0000'B | Plain NAS message, not security protected |  |
| Identity request message identity |  | '0101 0101'B | Identity request |  |
| Identity type |  | '0001'B | IMSI |  |
| Spare half octet |  | '0000'B |  |  |

NOTE:	 This message is sent without integrity protection before NAS security mode control procedure has been successfully completed and sent within SECURITY PROTECTED NAS MESSAGE message after NAS security mode control procedure has been successfully completed.

#### -	IDENTITY RESPONSE

This message is sent by the UE to the SS.

Table 4.7.2-18: IDENTITY RESPONSE

| Derivation Path: 24.301 clause 8.2.19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | EMM |  |  |
| Security header type |  | '0000'B | Plain NAS message, not security protected |  |
| Identity response message |  | '0101 0110'B | Identity response |  |
| Mobile identity |  | IMSI of the UE |  |  |

NOTE:	This message is sent without integrity protection before NAS security mode control procedure has been successfully completed and sent within SECURITY PROTECTED NAS MESSAGE message after NAS security mode control procedure has been successfully completed.

#### -	SECURITY MODE COMMAND

This message is sent by the SS to the UE.

Table 4.7.2-19: SECURITY MODE COMMAND

| Derivation Path: 24.301 clause 8.2.20 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | EMM |  |  |
| Security header type |  | '0000'B | Plain NAS message, not security protected |  |
| Security mode command message identity |  | '0101 1101'B | Security mode command |  |
| Selected NAS security algorithms |  |  |  |  |
| Type of integrity protection algorithm |  | Set according to PIXIT parameter for default integrity protection algorithm |  | For SIG |
|  |  | 0(reserved or eia0) | This IE is set to one of the algorithms supported by the UE as indicated in the IE "UE network capability" in the ATTACH REQUEST message. | For RF |
|  |  | eia1 |  |  |
|  |  | eia2 |  |  |
| Type of ciphering algorithm |  | Set according to PIXIT parameter for default ciphering algorithm |  | For SIG |
|  |  | eea0 |  | For RF |
| NAS key set identifier |  |  |  |  |
| NAS key set identifier |  | The valid NAS key set identifier. |  |  |
| TSC |  | '0'B | native security context (for KSIASME) |  |
| Spare half octet |  | '0000'B |  |  |
| Replayed UE security capabilities |  | Set according to the received UE security capabilities |  |  |
| IMEISV request |  | Not present |  |  |
| Replayed nonceUE |  | Not present |  |  |
| NonceMME |  | Not present |  |  |
| HashMME |  | Not present |  |  |
| Replayed UE additional security capability |  | Set to the value received in the Attach/TAU Request |  | EN-DC_duringRegistration |

| Condition | Explanation |
| --- | --- |
| For SIG | Used for signalling test cases |
| For RF | Used for RF/RRM test cases |
| EN-DC_duringRegistration | If the UE supports E-UTRA-NR dual connectivity NR ([71] pc_EN_DC) and this message is sent during an Attach or TAU procedure |

NOTE:	This message is always sent integrity protected with new EPS security context.

#### -	SECURITY MODE COMPLETE

This message is sent by the UE to the SS.

Table 4.7.2-20: SECURITY MODE COMPLETE

| Derivation Path: 24.301 clause 8.2.21 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | EMM |  |  |
| Security header type |  | '0000'B | Plain NAS message, not security protected |  |
| Security mode complete message identity |  | '0101 1110'B | Security mode complete |  |
| IMEISV |  | Not present |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message with new EPS security context.

#### -	SECURITY MODE REJECT

This message is sent by the UE to the SS.

Table 4.7.2-21: SECURITY MODE REJECT

| Derivation Path: 24.301 clause 8.2.22 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | EMM |  |  |
| Security header type |  | '0000'B | Plain NAS message, not security protected |  |
| Security mode reject message identity |  | '0101 1111'B | Security mode reject |  |
| EMM cause |  | The value is set according to specific message content. |  |  |

NOTE:	This message is sent without integrity protection before NAS security mode control procedure has been successfully completed and sent within SECURITY PROTECTED NAS MESSAGE message after NAS security mode control procedure has been successfully completed.

#### -	SERVICE ACCEPT

This message is sent by the SS to the UE.

Table 4.7.2-21A: SERVICE ACCEPT

| Derivation Path: 24.301 clause 8.2.34 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | EMM |  |  |
| Security header type |  | '0000'B | Plain NAS message, not security protected |  |
| Service accept message identity |  | '0100 1111'B | Service accept |  |
| EPS bearer context status |  | The same value as the value set in CONTROL PLANE SERVICE REQUEST |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

#### -	SERVICE REJECT

This message is sent by the SS to the UE.

Table 4.7.2-22: SERVICE REJECT

| Derivation Path: 24.301 clause 8.2.24 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | EMM |  |  |
| Security header type |  | '0000'B | Plain NAS message, not security protected |  |
| Service reject message identity |  | '0100 1110'B | Service reject |  |
| EMM cause |  | Set according to specific message content. |  |  |
| T3442 value |  | Not present |  |  |

NOTE:	If this message includes the EMM cause set to #3, #6, #7, #8, #11, #12, #13, #14, #15, #25 or #35, it is sent within SECURITY PROTECTED NAS MESSAGE message. Otherwise this message is sent without integrity protection.

#### -	SERVICE REQUEST

This message is sent by the UE to the SS.

Table 4.7.2-23: SERVICE REQUEST

| Derivation Path: 24.301 clause 8.2.25 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | EMM |  |  |
| Security header type |  | '1100'B | Security header for the SERVICE REQUEST message |  |
| KSI and sequence number |  |  |  |  |
| Sequence number (short) |  | The 5 least significant bits of the NAS COUNT value applicable when this message is sent for sequence number. |  |  |
| KSI |  | The effective KSI value. |  |  |
| Message authentication code (short) |  | The 2 least significant octets of the resulting message authentication code |  |  |

#### -	TRACKING AREA UPDATE ACCEPT

This message is sent by the SS to the UE.

Table 4.7.2-24: TRACKING AREA UPDATE ACCEPT

| Derivation Path: 24.301 clause 8.2.26 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | EMM |  |  |
| Security header type |  | '0000'B | Plain NAS message, not security protected |  |
| Tracking area update accept message identity |  | '0100 1001'B | Tracking area update accept |  |
| EPS update result |  | '0000'B | TA updated | TA_only |
|  |  | '0001'B | combined TA/LA updated | combined_TA_LA |
| Spare half octet |  | '0000'B |  |  |
| T3412 value |  | Not present |  | Periodic |
| T3412 value |  |  |  |  |
| Timer value |  | '0 0000'B |  |  |
| Unit |  | '111'B | value indicates that the timer is deactivated. |  |
| GUTI |  |  |  |  |
| Length of EPS mobile identity contents |  | '0000 1011'B | 11 octets |  |
| Type of identity |  | '110'B | GUTI |  |
| Odd/even indication |  | '0'B | even number of identity digits and also when the GUTI is used |  |
| MCC |  | See table 4.4.2-2 in this document | For NAS test cases, see table 6.3.2.2-1. |  |
| MNC |  | See table 4.4.2-2 in this document | For NAS test cases, see table 6.3.2.2-1. |  |
| MME Group ID |  | See table 4.4.2-2 in this document | For NAS test cases, see table 6.3.2.2-1. |  |
| MME Code |  | See table 4.4.2-2 in this document | For NAS test cases, see table 6.3.2.2-1. |  |
| M-TMSI |  | See table 4.4.2-2 in this document | For NAS test cases, see table 6.3.2.2-1. |  |
| TAI list |  |  |  |  |
| Length of tracking area identity list contents |  | '0000 0110'B | 6 octets |  |
| Partial tracking area identity list 1 |  |  |  |  |
| Number of elements |  | '0 0000'B | 1 element |  |
| Type of list |  | '00'B | list of TACs belonging to one PLMN, with non-consecutive TAC values |  |
| MCC |  | See table 4.4.2-2 in this document | For NAS test cases, see table 6.3.2.2-1. |  |
| MNC |  | See table 4.4.2-2 in this document | For NAS test cases, see table 6.3.2.2-1. |  |
| TAC 1 |  | See table 4.4.2-2 in this document | For NAS test cases, see table 6.3.2.2-1. |  |
| EPS bearer context status |  | The same value as the value set in TRACKING AREA UPDATE REQUEST message |  |  |
| Location area identification |  | Not present |  | TA_only |
| Location area identification |  |  |  | combined_TA_LA |
| MCC |  | MCC of the EUTRA cell from which this message is sent |  |  |
| MNC |  | MCC of the EUTRA cell from which this message is sent |  |  |
| LAC |  | 1 |  |  |
| MS identity |  | Not present |  | TA_only |
| MS identity |  |  |  | combined_TA_LA |
| Length of mobile identity contents |  | '0000 0101'B | 5 octets |  |
| Type of identity |  | '100'B | TMSI/P-TMSI/M-TMSI |  |
| Odd/even indication |  | '0'B | even number of identity digits and also when the TMSI/P-TMSI or TMGI and optional MBMS Session Identity is used |  |
| TMSI |  | TMSI-1 |  |  |
| EMM cause |  | Not present |  |  |
| T3402 value |  | Not present |  |  |
| T3423 value |  | Not present |  |  |
| Equivalent PLMNs |  | Not present |  |  |
| Emergency number list |  | Not present |  |  |
| EPS network feature support |  | 0000 0001'B | IMS voice over PS session in S1 mode supported |  |
| EPS network feature support |  | ‘1100 0000 0000 1100’B | CIoT control plane optimization indicated, Attach Without PDN supported, ePCO supported, Header Compression for CIoT control plane supported | (CIoT_CP OR CIoT_CP_SMSOnly) AND pc_NB |
| EPS network feature support |  | ‘1000 0000 0000 1011’B | CP CIoT supported, ePCO supported, S1-U data transfer supported, CIoT user plane optimization indicated | CIoT_UP AND pc_NB |
| EPS network feature support |  | ‘1000 0000 0000 1010’B | CP CIoT supported, ePCO supported, S1-U data transfer Only indicated | S1-U Only AND pc_NB |
| EPS network feature support |  | '0000 0011 0000 1000'B | - IMS voice over PS session in S1 mode supported - emergency bearer services in S1 mode supported - ePCO supported - Interworking without N26 interface not supported | NR |
| Additional update result |  | Not present |  |  |
| Additional update result |  | ‘0010’ B | “SMS only” | TAU_additional_update_result_SMS OR CIoT_CP_SMSOnly |
| T3412 extended value |  | Not present |  | Rel-10 |
| T3324 value |  | Not present |  |  |
| Extended DRX parameters |  | Not present |  |  |
| Header compression configuration status |  | Not present or set according to specific message content |  |  |
| DCN-ID |  | Not present |  |  |
| SMS services status |  | Not present |  |  |
| Non-3GPP NW provided policies |  | Not present |  |  |
| T3448 value |  | Not present |  |  |
| Network policy |  | Not present |  |  |
| T3447 value |  | Not present |  |  |
| Extended emergency number list |  | Not present |  |  |
| Ciphering key data |  | Not present |  |  |

| Condition | Explanation |
| --- | --- |
| TA_only | This condition applies if the UE is configured to initiate EPS attach or if explicitly specified. |
| combined_TA_LA | This condition applies if the UE is configured to initiate combined EPS/IMSI attach or if explicitly specified. This condition does not apply for NB-S1. |
| Periodic | This condition applies if in the last TRACKING AREA UPDATE REQUEST sent prior to this message, the EPS update type Value = '011'B (periodic updating). |
| TAU_additional_update_result_SMS | If the UE requested "SMS only" in the Additional update type IE and combined_TA_LA. |
| CIoT_CP | If test case requires control plane CIoT optimization |
| CIoT_CP_SMSOnly | If test case requires control plane CIoT optimization and SMS Only is indicated |
| CIoT_UP | If test case requires user plane CIoT optimization |
| S1-U data Only | If S1-U data transfer Only is indicated |
| NR | Used in NR test cases (TS 38.523-1 [71]) |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

#### -	TRACKING AREA UPDATE COMPLETE

This message is sent by the UE to the SS.

Table 4.7.2-25: TRACKING AREA UPDATE COMPLETE

| Derivation Path: 24.301 clause 8.2.27 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | EMM |  |  |
| Security header type |  | '0000'B | Plain NAS message, not security protected |  |
| Tracking area update complete message identity |  | '0100 1010'B | Tracking area update complete |  |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

#### -	TRACKING AREA UPDATE REJECT

This message is sent by the SS to the UE.

Table 4.7.2-26: TRACKING AREA UPDATE REJECT

| Derivation Path: 24.301 clause 8.2.28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | EMM |  |  |
| Security header type |  | '0000'B | Plain NAS message, not security protected |  |
| Tracking area update reject message identity |  | '0100 1011'B | Tracking area update reject |  |
| EMM cause |  | Set according to specific message content. |  |  |

NOTE:	If this message includes the EMM cause set to #3, #6, #7, #8, #11, #12, #13, #14, #15, #25 or #35, it is sent within SECURITY PROTECTED NAS MESSAGE message. Otherwise this message is sent without integrity protection.

#### -	TRACKING AREA UPDATE REQUEST

This message is sent by the UE to the SS.

Table 4.7.2-27: TRACKING AREA UPDATE REQUEST

| Derivation Path: 24.301 clause 8.2.29 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | EMM |  |  |
| Security header type |  | '0000'B | Plain NAS message, not security protected |  |
| Tracking area update request message identity |  | '0100 1000'B | Tracking area update request |  |
| EPS update type |  |  |  |  |
| EPS update type Value |  | '000'B | TA updating | TA_only |
|  |  | '001'B | Combined TA/LA updating | combined_TA_LA |
| "Active" flag |  | '0'B | No bearer establishment requested |  |
| NAS key set identifier |  |  |  |  |
| NAS key set identifier |  | The valid NAS key set identifier of the UE |  |  |
| TSC |  | '0'B | native security context (for KSIASME) |  |
| Old GUTI |  | Any allowed value |  |  |
| Non-current native NAS key set identifier |  | Not present |  |  |
| GPRS ciphering key sequence number |  | Not present |  |  |
| Old P-TMSI signature |  | Not present |  |  |
| Additional GUTI |  | Not present |  |  |
| NonceUE |  | Not present |  |  |
| UE network capability |  | Not present or any allowed value |  |  |
| UE network capability |  |  |  | EN-DC |
| All octets with the exception of octet 9, bit 5 |  | Any allowed value |  |  |
| Dual connectivity with NR (DCNR) (octet 9, bit 5) |  | 1 | dual connectivity with NR supported |  |
| UE network capability |  |  |  | NR |
| All octets with the exception of octet 8, bit 8 and octet 9, bit 6 |  | Any allowed value |  |  |
| Extended protocol configuration options (ePCO) (octet 8, bit 8) |  | 1 | ePCO supported |  |
| N1 mode supported (N1mode) (octet 9, bit 6) |  | 1 | N1 mode supported |  |
| Last visited registered TAI |  | Not present or any allowed value |  |  |
| DRX parameter |  | Not present or any allowed value |  |  |
| UE radio capability information update needed |  | Not present or any allowed value |  |  |
| EPS bearer context status |  | Not present or (octet 3 = '00100000'B and octet 4 = '00000000'B) |  |  |
| MS network capability |  | Not present or any allowed value |  |  |
| Old location area identification |  | Not present or any allowed value |  |  |
| TMSI status |  | Not present or any allowed value |  |  |
| Mobile station classmark 2 |  | Not present or any allowed value |  |  |
| Mobile station classmark 3 |  | Not present or any allowed value |  |  |
| Supported Codecs |  | Not present or any allowed value |  |  |
| Additional update type |  | Not present |  | TA_only |
| Additional update type |  | Not present or any allowed value |  | combined_TA_LA |
| Additional update type |  | Any allowed value |  | TA_only_with_SMS |
| Additional update type |  | Not present or any allowed value |  | CIoT |
| Voice domain preference and UE's usage setting |  | Not present or any allowed value |  |  |
| Old GUTI type |  | Not present or any allowed value |  |  |
| Device properties |  | Not present or any allowed value |  |  |
| MS network feature support |  | Not present or any allowed value |  |  |
| TMSI based NRI container |  | Not present or any allowed value |  |  |
| T3324 value |  | Not present or any allowed value |  |  |
| T3412 extended value |  | Not present or any allowed value |  |  |
| Extended DRX parameters |  | Not present or any allowed value |  |  |
| UE additional security capability |  | Any allowed value (NOTE 1, NOTE 2) |  | EN-DC OR NR |
| UE status |  | Not present or any allowed value |  |  |
| Additional information requested |  | Not present or any allowed value |  |  |
| NOTE 1: For a UE supporting dual connectivity with NR, if the UE supports one of the encryption algorithms for 5GS in bits 8 to 5 of octet 3, it shall support the same algorithms for E-UTRAN as specified in 3GPP TS 33.401 [41]. NOTE 2: For a UE supporting dual connectivity with NR, if the UE supports one of the integrity protection algorithms for 5GS in bits 8 to 5 of octet 5, it shall support the same algorithms for E-UTRAN as specified in 3GPP TS 33.401 [41]. |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| TA_only | See the definition below table 4.7.2-24. |
| combined_TA_LA | See the definition below table 4.7.2-24. |
| EPS_only_with_SMS | If the UE wants to update for EPS attach with SMS only for CIoT optimization |
| CIoT | If UE supports CIoT Optimisation |
| EN-DC | Used in E-UTRA-NR dual connectivity test cases (TS 38.523-1 [71]) |
| NR | Used in NR test cases (TS 38.523-1 [71]) |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

#### -	UPLINK NAS TRANSPORT

This message is sent by the UE to the SS.

Table 4.7.2-27A: UPLINK NAS TRANSPORT

| Derivation Path: 24.301 clause 8.2.30 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | EMM |  |  |
| Security header type |  | '0000'B | Plain NAS message, not security protected |  |
| Uplink NAS transport message identity |  | '0100 0011'B | Uplink NAS transport |  |
| NAS message container |  | Set according to specific message content |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

### 4.7.3	Contents of ESM messages

#### -	ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT

This message is sent by the UE to the SS.

Table 4.7.3-1: ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT

| Derivation Path: 24.301 clause 8.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | ESM |  |  |
| EPS bearer identity |  | The same value as the value set in ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message |  |  |
| Procedure transaction identity |  | '0000 0000'B | No procedure transaction identity assigned |  |
| Activate dedicated EPS bearer context accept message identity |  | '1100 0110'B | Activate dedicated EPS bearer context accept |  |
| Protocol configuration options |  | Not present or any allowed value |  | NOT (pc_NB or pc_NonIP_PDN) |
| Protocol configuration options |  | Not present |  | pc_NB or pc_NonIP_PDN |
| NBIFOM container |  | Not present or any allowed value |  |  |
| Extended protocol configuration options |  | Not present or any allowed value |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

#### -	ACTIVATE DEDICATED EPS BEARER CONTEXT REJECT

This message is sent by UE to the SS.

Table 4.7.3-2: ACTIVATE DEDICATED EPS BEARER CONTEXT REJECT

| Derivation Path: 24.301 clause 8.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | ESM |  |  |
| EPS bearer identity |  | The same value as the value set in ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message. |  |  |
| Procedure transaction identity |  | '0000 0000'B | No procedure transaction identity assigned |  |
| Activate dedicated EPS bearer context reject message identity |  | '1100 0111'B | Activate dedicated EPS bearer context reject |  |
| ESM cause |  | The value is set according to specific message content. |  |  |
| Protocol configuration options |  | Not present |  |  |
| NBIFOM container |  | Not present or any allowed value |  |  |
| Extended protocol configuration options |  | Not present or any allowed value |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

#### -	ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST

This message is sent by the SS to the UE.

Table 4.7.3-3: ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST

| Derivation Path: 24.301 clause 8.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | ESM |  |  |
| EPS bearer identity |  | Arbitrarily selected value between '0101'B and '1111'B. |  |  |
| Procedure transaction identity |  | '0000 0000'B | No procedure transaction identity assigned | NETWORK-INITIATED |
|  |  | The same value as the value set in BEARER RESOURCE MODIFICATION REQUEST message or BEARER RESOURCE ALLOCATION REQUEST message |  | UE-INITIATED |
| Activate dedicated EPS bearer context request message identity |  | '1100 0101'B | Activate dedicated EPS bearer context request |  |
| Linked EPS bearer identity |  | The EPS bearer identity of the associated default bearer |  |  |
| Spare half octet |  | '0000'B |  |  |
| EPS QoS |  | See Reference dedicated EPS bearer context #1 in table 6.6.2-1 |  |  |
| TFT |  | See Reference dedicated EPS bearer context #1 in table 6.6.2-1 |  |  |
| Transaction identifier |  | Not present |  |  |
|  |  | Distinct value between 0 and 127 calculated on basis of the EPS bearer identity. |  | pc_UTRAN AND/OR pc_GERAN |
| Negotiated QoS |  | See Reference dedicated EPS bearer context #1 in table 6.6.2-1 |  |  |
| Negotiated LLC SAPI |  | See Reference dedicated EPS bearer context #1 in table 6.6.2-1 |  |  |
| Radio priority |  | See Reference dedicated EPS bearer context #1 in table 6.6.2-1 |  |  |
| Packet flow Identifier |  | Not present |  |  |
|  |  | '0000 0000'B | Best Effort | pc_GERAN |
| Protocol configuration options |  | See Reference dedicated EPS bearer context #1 in table 6.6.2-1 |  | NOT (pc_NB or pc_NonIP_PDN) |
| Protocol configuration options |  | Not present |  | pc_NB or pc_NonIP_PDN |
| WLAN offload container |  | Not present |  |  |
| NBIFOM container |  | Not present |  |  |
| Extended protocol configuration options |  | Not present |  | NOT (pc_NB or pc_NonIP_PDN) |
| Extended protocol configuration options |  | See Reference dedicated EPS bearer context #1 in table 6.6.2-1 |  | pc_NB or pc_NonIP_PDN |

| Condition | Explanation |
| --- | --- |
| NETWORK-INITIATED | Network initiated ESM procedures |
| UE-INITIATED | UE initiated ESM procedures |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

#### -	ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT

This message is sent by the UE to the SS.

Table 4.7.3-4: ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT

| Derivation Path: 24.301 clause 8.3.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | ESM |  |  |
| EPS bearer identity |  | The same value as the value set in ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message |  |  |
| Procedure transaction identity |  | '0000 0000'B | No procedure transaction identity assigned |  |
| Activate default EPS bearer context accept message identity |  | '1100 0010'B | Activate default EPS bearer context accept |  |
| Protocol configuration options |  | Not present or any allowed value |  | NOT (pc_NB or pc_NonIP_PDN) |
| Protocol configuration options |  | Not present |  | pc_NB or pc_NonIP_PDN |
| Extended protocol configuration options |  | Not present or any allowed value |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

#### -	ACTIVATE DEFAULT EPS BEARER CONTEXT REJECT

This message is sent by UE to the SS.

Table 4.7.3-5: ACTIVATE DEFAULT EPS BEARER CONTEXT REJECT

| Derivation Path: 24.301 clause 8.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | ESM |  |  |
| EPS bearer identity |  | The same value as the value set in ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message. |  |  |
| Procedure transaction identity |  | '0000 0000'B | No procedure transaction identity assigned |  |
| Activate default EPS bearer context reject message identity |  | '1100 0011'B | Activate default EPS bearer context reject |  |
| ESM cause |  | The value is set according to specific message content. |  |  |
| Protocol configuration options |  | Not present |  |  |
| Extended protocol configuration options |  | Not present |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

#### -	ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST

This message is sent by the SS to the UE.

Table 4.7.3-6: ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST

| Derivation Path: 24.301 clause 8.3.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | ESM |  |  |
| EPS bearer identity |  | An arbitrarily selected value between '0101'B and '1111'B. |  |  |
| Procedure transaction identity |  | The same value as the value set in the latest PDN CONNECTIVITY REQUEST message sent prior to this message. |  |  |
| Activate default EPS bearer context request message identity |  | '1100 0001'B | Activate default EPS bearer context request |  |
| EPS QoS |  | See Reference default EPS bearer context #1 in table 6.6.1-1 |  | NOT IMS_PDN_ConnEstab |
| EPS QoS |  | See Reference default EPS bearer context #2 in table 6.6.1-1 |  | IMS_PDN_ConnEstab |
| Access point name |  | The SS defines a Default APN |  | NOT IMS_PDN_ConnEstab AND NOT APN_Provided |
| Access point name |  | APN as provided in ESM INFORMATION RESPONSE or PDN CONNECTIVITY REQUEST |  | NOT IMS_PDN_ConnEstab AND APN_Provided |
| Access point name |  | IMS. mnc<MNC>.mcc<MCC>.gprs The <MNC> and <MCC> are set to the same values as in IMSI. |  | IMS_PDN_ConnEstab AND NOT APN_Provided |
| Access point name |  | Use APN Network Identifier provided in ESM INFORMATION RESPONSE or PDN CONNECTIVITY REQUEST message and the APN Operator Identifier mnc<MNC>.mcc<MCC>.gprs. The <MNC> and <MCC> are set to the same values as in IMSI. |  | IMS_PDN_ConnEstab AND APN_Provided |
| PDN address |  |  |  | IPv4 |
| Length of PDN address contents |  | 5 octets |  |  |
| PDN type value |  | ‘001’B | IPv4 |  |
| PDN address information |  | IPv4 address | The SS provides a valid IPv4 address | NOT IPv4-DHCP |
|  |  | 0.0.0.0 | DHCPv4 is to be used to allocate the IPv4 address | IPv4-DHCP |
| PDN address |  |  |  | IPv6 |
| Length of PDN address contents |  | 9 octets |  |  |
| PDN type value |  | ‘010’B | IPv6 |  |
| PDN address information |  | IPv6 interface identifier | The SS provides a valid IPv6 interface identifier |  |
| PDN address |  |  |  | IPv4v6 |
| Length of PDN address contents |  | 13 octets |  |  |
| PDN type value |  | ‘011’B | IPv4v6 |  |
| PDN address information (Octets 4 to 11) |  | IPv6 interface identifier | The SS provides a valid IPv6 interface identifier |  |
| PDN address information (Octets 12 to 15) |  | IPv4 address | The SS provides a valid IPv4 address | NOT IPv4-DHCP |
|  |  | 0.0.0.0 | DHCPv4 is to be used to allocate the IPv4 address | IPv4-DHCP |
| PDN address |  |  |  | CIoT_CP_NON_IP |
| Length of PDN address contents |  | 2 octets |  |  |
| PDN type value |  | ‘101’B | Non-IP PDN |  |
| PDN address information (Octets 4 to 7) |  | Coded as 0 | spare |  |
| Transaction identifier |  | Not present |  |  |
| Transaction identifier |  | Distinct value between 0 and 127 calculated on basis of the EPS bearer identity. |  | pc_UTRAN AND/OR pc_GERAN |
| Negotiated QoS |  | See Reference default EPS bearer context #1 in table 6.6.1-1 |  |  |
| Negotiated LLC SAPI |  | See Reference default EPS bearer context #1 in table 6.6.1-1 |  |  |
| Radio priority |  | See Reference default EPS bearer context #1 in table 6.6.1-1 |  |  |
| Packet flow Identifier |  | Not present |  |  |
| Packet flow Identifier |  | '0000 0000'B | Best Effort | pc_GERAN |
| APN-AMBR |  | See Reference default EPS bearer context #1 in table 6.6.1-1 |  |  |
| ESM cause |  | Not present |  |  |
| Protocol configuration options |  | Not present |  | pc_NB or pc_NonIP_PDN |
| Protocol configuration options |  |  |  | NOT (pc_NB or pc_NonIP_PDN) |
| Configuration protocol |  | See Reference default EPS bearer context #1 in table 6.6.1-1 |  |  |
| Container ID 1 |  | ‘0001’H |  | P-CSCF IPv6 |
| Length of container ID 1 contents |  |  | Length value determined by the TTCN implementation |  |
| Container ID 1 contents |  | IPv6 address | P-CSCF IPv6 Address |  |
| Container ID 2 |  | ‘000C’H |  | P-CSCF IPv4 |
| Length of container ID 2 contents |  |  | Length value determined by the TTCN implementation |  |
| Container ID 2 contents |  | IPv4 address | P-CSCF IPv4 Address |  |
| Container ID n |  | ‘0003’H | n assigned to next available number | DNS IPv6 |
| Length of container ID n contents |  |  | Length value determined by the TTCN implementation |  |
| Container ID n contents |  | IPv6 address | DNS IPv6 Address |  |
| Container ID n+1 |  | ‘000D’H | n assigned to next available number | DNS IPv4 |
| Length of container ID n+1 contents |  |  | Length value determined by the TTCN implementation |  |
| Container ID n+1 contents |  | IPv4 address | DNS IPv4 Address |  |
| Container ID n+2 |  | ‘001D’H | n assigned to next available number | Interworking_with_5GS |
| Length of container ID n+2 contents |  |  | Length value determined by the TTCN implementation |  |
| Container ID n+2 contents |  | Session-AMBR | Session-AMBR |  |
| Session AMBR |  |  |  |  |
| Unit for Session-AMBR for downlink |  | ‘000 00101’ | Value is incremented in multiples of 256 Kbps |  |
| Session-AMBR for downlink |  | ‘0000 0000 0000 0100’B | 1024 Kbps |  |
| Unit for Session-AMBR for uplink |  | ‘000 00101’ | Value is incremented in multiples of 256 Kbps |  |
| Session-AMBR for uplink |  | ‘0000 0000 0000 0100’B | 1024 Kbps |  |
| Container ID n+3 |  | ‘001C’H | n assigned to next available number | Interworking_with_5GS |
| Length of container ID n+3 contents |  |  | Length value determined by the TTCN implementation |  |
| Container ID n+3 contents |  | QoS rules | QoS rules according to TS 38.508-1 [67] Table 4.8.2.1-1 | NOT IMS_PDN_ConnEstab |
| Container ID n+3 contents |  | QoS rules | QoS rules according to TS 38.508-1 [67] Table 4.8.2.1-2 | IMS_PDN_ConnEstab |
| Container ID n+4 |  | ‘001F’H | n assigned to next available number | Interworking_with_5GS |
| Length of container ID n+4 contents |  |  | Length value determined by the TTCN implementation |  |
| Container ID n+4 contents |  | Qos flow descriptions | Qos flow descriptions according to TS 38.508-1 [67] 4.8.2.3-1 with condition Interworking_with_EPS | NOT IMS_PDN_ConnEstab |
| Container ID n+4 contents |  | Qos flow descriptions | Qos flow descriptions refer to TS 38.508-1 [67] 4.8.2.3-2 with condition Interworking_with_EPS | IMS_PDN_ConnEstab |
| Container ID n+5 |  | ‘001B’H | n assigned to next available number | Interworking_with_5GS |
| Length of container ID n+5 contents |  |  | Length value determined by the TTCN implementation |  |
| Container ID n+5 contents (Octet 1) |  | ‘0000 0001’B | SST of S-NSSAI same as the allowed NSSAI in TS 38.508-1 [67] Table 4.7.1-7: REGISTRATION ACCEPT |  |
| Container ID n+5 contents (Octets 2 to 4) |  | PLMN ID | PLMD ID same as TS 38.508-1 [67] Table 6.3.2.2-1 |  |
| WLAN offload indication |  | Not present |  |  |
| WLAN offload indication |  |  |  | WLAN |
| WLAN offload acceptability |  | ‘0001’B | Offloading the traffic of the PDN connection via a WLAN when in S1 mode is acceptable |  |
| NBIFOM container |  | Not present |  |  |
| Header compression configuration |  | Not present or set according to specific message content |  |  |
| Control plane only indication |  | Not present |  |  |
| Control plane only indication |  | ‘0001’B |  | CIoT_ControlPlane_PDN |
| Extended protocol configuration options |  | Not present |  | NOT (pc_NB or pc_NonIP_PDN) |
| Extended protocol configuration options |  |  |  | pc_NB or pc_NonIP_PDN |
| Configuration protocol |  | See Reference default EPS bearer context #1 in table 6.6.1-1 |  |  |
| Container ID n |  | ‘0003’H | n assigned to next available number | DNS IPv6 |
| Length of container ID n contents |  |  | Length value determined by the TTCN implementation |  |
| Container ID n contents |  | IPv6 address | DNS IPv6 Address |  |
| Container ID n+1 |  | ‘000D’H | n assigned to next available number | DNS IPv4 |
| Length of container ID n+1 contents |  |  | Length value determined by the TTCN implementation |  |
| Container ID n+1 contents |  | IPv4 address | DNS IPv4 Address |  |
| Serving PLMN rate control |  | Not present |  |  |
| Serving PLMN rate control |  | ‘FFFF’H | Maximum number of ESM DATA TRANSPORT messages allowed per 6 min interval is not restricted | CIoT_ControlPlane_PDN |

| Condition | Explanation |
| --- | --- |
| IPv4 | If in the last PDN CONNECTIVITY REQUEST sent prior to this message, the PDN type = '001'B |
| IPv6 | If in the last PDN CONNECTIVITY REQUEST sent prior to this message, the PDN type = '010'B |
| IPv4v6 | If in the last PDN CONNECTIVITY REQUEST sent prior to this message, the PDN type = '011'B |
| IPv4-DHCP | If in the last PDN CONNECTIVITY REQUEST or ESM INFORMATION RESPONSE sent prior to this message, the IE Protocol configuration options contains a configuration protocol option = '000B00H' ("IPv4 address allocation via DHCPv4", length of contents = 0). Note 1: This condition is used in conjunction with IPv4 or IPv4v6 as indicated in the "PDN address row" just above. Note 2: If both messages, PDN CONNECTIVITY REQUEST and ESM INFORMATION RESPONSE, are received and contain a Protocol configuration options IE then the IE from the message which is received later shall be used. |
| P-CSCF IPv6 | If in the last PDN CONNECTIVITY REQUEST or ESM INFORMATION RESPONSE sent prior to this message, the Protocol configuration options and the additional parameter list was included with a “P-CSCF IPv6 Address Request” Note 1: This condition is only applicable for UEs with IMS support (TS 36.523-2 A.4.4-1/25). Note 2: If both messages, PDN CONNECTIVITY REQUEST and ESM INFORMATION RESPONSE, are received and contain a Protocol configuration options IE then the IE from the message which is received later shall be used. |
| P-CSCF IPv4 | If in the last PDN CONNECTIVITY REQUEST or ESM INFORMATION RESPONSE sent prior to this message, the Protocol configuration options and the additional parameter list was included with a “P-CSCF IPv4 Address Request” Note 1: This condition is only applicable for UEs with IMS support (TS 36.523-2 A.4.4-1/25). Note 2: If both messages, PDN CONNECTIVITY REQUEST and ESM INFORMATION RESPONSE, are received and contain a Protocol configuration options IE then the IE from the message which is received later shall be used. |
| DNS IPv6 | If in the last PDN CONNECTIVITY REQUEST or ESM INFORMATION RESPONSE sent prior to this message, the Protocol configuration options and the additional parameter list was included with a “DNS IPv6 Address Request”. Note: If both messages, PDN CONNECTIVITY REQUEST and ESM INFORMATION RESPONSE, are received and contain a Protocol configuration options IE then the IE from the message which is received later shall be used. |
| DNS IPv4 | If in the last PDN CONNECTIVITY REQUEST or ESM INFORMATION RESPONSE sent prior to this message, the Protocol configuration options and the additional parameter list was included with a “DNS IPv4 Address Request”. Note: If both messages, PDN CONNECTIVITY REQUEST and ESM INFORMATION RESPONSE, are received and contain a Protocol configuration options IE then the IE from the message which is received later shall be used. |
| APN_Provided | If in the last PDN CONNECTIVITY REQUEST or ESM INFORMATION RESPONSE sent prior to this message an Access Point Name was included NOTE: APN_Provided is equivalent to pc_Provide_IMS_APN for establishment of the first PDN connectivity during initial registration and if the first PDN is IMS (see table 4.7.3-14) |
| IMS_PDN_ConnEstab | If this message refers to establishment of connectivity to the IMS PDN (only applicable if the UE supports IMS according to TS 36.523-2 A.4.4-1/25). |
| WLAN | If the message is being sent for RAN Assisted WLAN interworking. |
| CIoT_ControlPlane_PDN | If the associated PDN connection is only for control plane CIoT optimization |
| CIoT_CP_NON_IP | If the UE requests a Non-IP type PDN |
| Interworking_with_5GS | If the UE has indicated support of N1, then the SS shall include these IEs to provide details for the interworking with 5GS. Applicable to 5G test cases defined in TS 38.523-1 [71]. |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

#### -	BEARER RESOURCE ALLOCATION REJECT

This message is sent by the SS to the UE.

Table 4.7.3-6A: BEARER RESOURCE ALLOCATION REJECT

| Derivation Path: 24.301 clause 8.3.7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | ESM |  |  |
| EPS bearer identity |  | '0000'B | No EPS bearer identity assigned |  |
| Procedure transaction identity |  | The value indicated in BEARER RESOURCE ALLOCATION REQUEST message. |  |  |
| Bearer resource allocation reject message identity |  | '1101 0101'B | Bearer resource allocation reject |  |
| ESM cause |  | Set according to specific message content. |  |  |
| Protocol configuration options |  | Not present |  |  |
| NBIFOM container |  | Not present |  |  |
| Extended protocol configuration options |  | Not present |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

#### -	BEARER RESOURCE ALLOCATION REQUEST

This message is sent by the UE to the SS.

Table 4.7.3-6B: BEARER RESOURCE ALLOCATION REQUEST

| Derivation Path: 24.301 clause 8.3.8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | ESM |  |  |
| EPS bearer identity |  | '0000'B | No EPS bearer identity assigned |  |
| Procedure transaction identity |  | Any value from 1 to 254 |  |  |
| Bearer resource allocation request message identity |  | '1101 0100'B | Bearer resource allocation request |  |
| Linked EPS bearer identity |  | The EPS bearer identity of the associated default bearer. |  |  |
| Spare half octet |  | '0000'B |  |  |
| Traffic flow aggregate |  |  |  |  |
| Number of packet filters |  | Greater than 0 and less than or equal to 16 |  |  |
| E bit |  | Any allowed value |  |  |
| TFT operation code |  | '001'B | Create new TFT |  |
| Packet filter list |  | Any allowed value |  |  |
| Parameters list |  | Not present or any allowed value |  |  |
| Required traffic flow QoS |  | Any allowed value |  |  |
| Protocol configuration options |  | Not present or any allowed value |  |  |
| NBIFOM container |  | Not present or any allowed value |  |  |
| Extended protocol configuration options |  | Not present or any allowed value |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

#### -	BEARER RESOURCE MODIFICATION REJECT

This message is sent by the SS to the UE.

Table 4.7.3-7: BEARER RESOURCE MODIFICATION REJECT

| Derivation Path: 24.301 clause 8.3.9 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | ESM |  |  |
| EPS bearer identity |  | '0000'B | No EPS bearer identity assigned |  |
| Procedure transaction identity |  | The value indicated in BEARER RESOURCE MODIFICATION REQUEST message. |  |  |
| Bearer resource modification reject message identity |  | '1101 0111'B | Bearer resource modification reject |  |
| ESM cause |  | Set according to specific message content. |  |  |
| Protocol configuration options |  | Not present |  |  |
| NBIFOM container |  | Not present |  |  |
| Extended protocol configuration options |  | Not present |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

#### -	BEARER RESOURCE MODIFICATION REQUEST

This message is sent by the UE to the SS.

Table 4.7.3-8: BEARER RESOURCE MODIFICATION REQUEST

| Derivation Path: 24.301 clause 8.3.10 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | ESM |  |  |
| EPS bearer identity |  | '0000'B | No EPS bearer identity assigned |  |
| Procedure transaction identity |  | Any value from 1 to 254 |  |  |
| Bearer resource modification request message identity |  | '1101 0110'B | Bearer resource modification request |  |
| EPS bearer identity for packet filter |  | The EPS bearer identity of the associated dedicated EPS bearer related with packet filter. |  |  |
| Spare half octet |  | '0000'B |  |  |
| Traffic flow aggregate |  | Any allowed value |  |  |
| Traffic flow aggregate |  |  |  | RELEASE-REQUESTED |
| Number of packet filters |  | Greater than 0 and less than or equal to 16 |  |  |
| E bit |  | Any allowed value |  |  |
| TFT operation code |  | '101'B | Delete packet filters from existing TFT |  |
| Packet filter list |  | Any allowed value |  |  |
| Parameters list |  | Not present or any allowed value |  |  |
| Required traffic flow QoS |  | Not present or any allowed value |  |  |
| ESM cause |  | Not present |  |  |
|  |  | '0010 0100'B | Regular deactivation | RELEASE-REQUESTED |
| Protocol configuration options |  | Not present or any allowed value |  |  |
| NBIFOM container |  | Not present or any allowed value |  |  |
| Header compression configuration |  | Not present or any allowed value |  |  |
| Extended protocol configuration options |  | Not present or any allowed value |  |  |

| Condition | Explanation |
| --- | --- |
| RELEASE-REQUESTED | UE requests the release of bearer resources. |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

Table 4.7.3-9: (Void)

Table 4.7.3-10: (Void)

#### -	DEACTIVATE EPS BEARER CONTEXT ACCEPT

This message is sent by the UE to the SS.

Table 4.7.3-11: DEACTIVATE EPS BEARER CONTEXT ACCEPT

| Derivation Path: 24.301 clause 8.3.9 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | ESM |  |  |
| EPS bearer identity |  | The same value as the value set in DEACTIVATE EPS BEARER CONTEXT REQUEST message. |  |  |
| Procedure transaction identity |  | '0000 0000'B | No procedure transaction identity assigned |  |
| Deactivate EPS bearer context accept message identity |  | '1100 1110'B | Deactivate EPS bearer context accept |  |
| Protocol configuration options |  | Not present or any allowed value |  |  |
| Extended protocol configuration options |  | Not present or any allowed value |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

#### -	DEACTIVATE EPS BEARER CONTEXT REQUEST

This message is sent by the SS to the UE.

Table 4.7.3-12: DEACTIVATE EPS BEARER CONTEXT REQUEST

| Derivation Path: 24.301 clause 8.3.10 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | ESM |  |  |
| EPS bearer identity |  | Set according to specific message content. |  |  |
| Procedure transaction identity |  | '0000 0000'B | No procedure transaction identity assigned | NETWORK-INITIATED |
|  |  | The same value as the value set in PDN DISCONNECT REQUEST message or BEARER RESOURCE MODIFICATION REQUEST message. |  | UE-INITIATED |
| Deactivate EPS bearer context request message identity |  | '1100 1101'B | Deactivate EPS bearer context request |  |
| ESM cause |  | Set according to specific message content. |  |  |
| Protocol configuration options |  | Not present |  |  |
| NBIFOM container |  | Not present |  |  |
| Extended protocol configuration options |  | Not present |  |  |

| Condition | Explanation |
| --- | --- |
| NETWORK-INITIATED | Network initiated ESM procedures |
| UE-INITIATED | UE initiated ESM procedures |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

#### -	ESM DATA TRANSPORT

This message is sent by the UE or by the SS.

Table 4.7.3-12A: ESM DATA TRANSPORT

| Derivation Path: 24.301 clause 8.3.25 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | ESM |  |  |
| EPS bearer identity |  | '0000'B | No EPS bearer identity assigned |  |
| Procedure transaction identity |  | Any value from 1 to 254 |  |  |
| ESM data transport message identity |  | '1110 1011'B | ESM data transport |  |
| User data container |  | set according to specific message content |  |  |
| Release assistance indication |  | Not present or set according to specific message content |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

#### -	ESM DUMMY MESSAGE

This message is sent by the SS to the UE or by the UE to the SS.

Table 4.7.3-12B: ESM DUMMY MESSAGE

| Derivation Path: 24.301 clause 8.3.12A |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | ESM |  |  |
| EPS bearer identity |  | '0000'B | No EPS bearer identity assigned |  |
| Procedure transaction identity |  | The value indicated in ESM DUMMY MESSAGE message. |  |  |
| ESM information request message identity |  | '1101 1100'B | ESM dummy message |  |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

#### -	ESM INFORMATION REQUEST

This message is sent by the SS to the UE.

Table 4.7.3-13: ESM INFORMATION REQUEST

| Derivation Path: 24.301 clause 8.3.11 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | ESM |  |  |
| EPS bearer identity |  | '0000'B | No EPS bearer identity assigned |  |
| Procedure transaction identity |  | The value indicated in PDN CONNECTIVITY REQUEST message. |  |  |
| ESM information request message identity |  | '1101 1001'B | ESM information request |  |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

#### -	ESM INFORMATION RESPONSE

This message is sent by the UE to the SS.

Table 4.7.3-14: ESM INFORMATION RESPONSE

| Derivation Path: 24.301 clause 8.3.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | ESM |  |  |
| EPS bearer identity |  | '0000'B | No EPS bearer identity assigned |  |
| Procedure transaction identity |  | The same value as the value set in ESM INFORMATION REQUEST message. |  |  |
| ESM information response message identity |  | '1101 1010'B | ESM information response |  |
| Access point name |  | Not present or any allowed value |  | Internet_APN |
| Access point name |  | Not present |  | IMS_APN_Default |
| Access point name |  | Any allowed value |  | IMS_APN_Provided |
| Protocol configuration options |  | Not present or any allowed value |  | NOT (pc_NB or pc_NonIP_PDN) |
| Protocol configuration options |  | If present including at least the following container |  | Interworking_with_5GS |
| Container ID n |  | ‘001A’H | n assigned to next available number |  |
| Length of container ID n contents |  |  | 1 octet |  |
| Container ID n contents |  | PDU session ID | PDU session ID assigned by the UE |  |
| Protocol configuration options |  | Not present |  | pc_NB or pc_NonIP_PDN |
| Extended protocol configuration options |  | Not present or any allowed value |  |  |
| Extended protocol configuration options |  | If present including at least the following container |  | Interworking_with_5GS |
| Container ID n |  | ‘001A’H | n assigned to next available number |  |
| Length of container ID n contents |  |  | 1 octet |  |
| Container ID n contents |  | PDU session ID | PDU session ID assigned by the UE |  |

| Condition | Expression | Explanation |
| --- | --- | --- |
| IMS_APN_Provided | pc_IMS AND NOT pc_Provide_IMS_as_second_APN AND pc_Provide_IMS_APN | IMS APN shall be provided by the UE: |
| IMS_APN_Default | pc_IMS AND NOT pc_Provide_IMS_as_second_APN AND NOT pc_Provide_IMS_APN | No IMS APN shall be provided by the UE; IMS APN is default APN |
| Internet_APN | NOT pc_IMS OR pc_Provide_IMS_as_second_APN |  |
| Interworking_with_5GS |  | If the UE has indicated support of N1, then the SS shall include this IE to provide details for the interworking with 5GS. Applicable to 5G test cases defined in TS 38.523-1 [71]. Note: The PDU session ID container shall be sent in either the Protocol configuration options or the Extended protocol configuration options IE. |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

#### -	ESM STATUS

This message is sent by the UE or by the SS.

Table 4.7.3-15: ESM STATUS

| Derivation Path: 24.301 clause 8.3.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | ESM |  |  |
| EPS bearer identity |  | Set according to specific message content. |  |  |
| Procedure transaction identity |  | '0000 0000'B | No procedure transaction identity assigned |  |
| ESM status message identity |  | '1110 1000'B | ESM status |  |
| ESM cause |  | Set according to specific message content. |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

#### -	MODIFY EPS BEARER CONTEXT ACCEPT

This message is sent by the UE to the SS.

Table 4.7.3-16: MODIFY EPS BEARER CONTEXT ACCEPT

| Derivation Path: 24.301 clause 8.3.14 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | ESM |  |  |
| EPS bearer identity |  | The same value as the value set in MODIFY EPS BEARER CONTEXT REQUEST message. |  |  |
| Procedure transaction identity |  | '0000 0000'B | No procedure transaction identity assigned |  |
| Modify EPS bearer context accept message identity |  | '1100 1010'B | Modify EPS bearer context accept |  |
| Protocol configuration options |  | Not present or any allowed value |  |  |
| NBIFOM container |  | Not present or any allowed value |  |  |
| Extended protocol configuration options |  | Not present or any allowed value |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

#### -	MODIFY EPS BEARER CONTEXT REJECT

This message is sent by the UE to the SS.

Table 4.7.3-17: MODIFY EPS BEARER CONTEXT REJECT

| Derivation Path: 24.301 clause 8.3.15 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | ESM |  |  |
| EPS bearer identity |  | The same value as the value set in MODIFY EPS BEARER CONTEXT REQUEST message. |  |  |
| Procedure transaction identity |  | '0000 0000'B | No procedure transaction identity assigned |  |
| Modify EPS bearer context reject message identity |  | '1100 1011'B | Modify EPS bearer context reject |  |
| ESM cause |  | The value is set according to specific message content. |  |  |
| Protocol configuration options |  | Not present |  |  |
| NBIFOM container |  | Not present |  |  |
| Extended protocol configuration options |  | Not present |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

#### -	MODIFY EPS BEARER CONTEXT REQUEST

This message is sent by the SS to the UE.

Table 4.7.3-18: MODIFY EPS BEARER CONTEXT REQUEST

| Derivation Path: 24.301 clause 8.3.16 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | ESM |  |  |
| EPS bearer identity |  | Set according to specific message content. |  |  |
| Procedure transaction identity |  | '0000 0000'B | No procedure transaction identity assigned | NETWORK-INITIATED |
|  |  | The same value as the value set in BEARER RESOURCE MODIFICATION REQUEST message or BEARER RESOURCE ALLOCATION REQUEST message. |  | UE-INITIATED |
| Modify EPS bearer context request message identity |  | '1100 1001'B | Modify EPS bearer context request |  |
| New EPS QoS |  | Use the same value as used in Activate EPS Bearer Context Request message |  |  |
| TFT |  | According to reference dedicated EPS bearer context #2 except for TFT operation code which is set to ‘100’B and TFT identifier is set to ‘00110000’B.. |  |  |
| New QoS |  | Not Present. |  |  |
| Negotiated LLC SAPI |  | Not Present. |  |  |
| Radio priority |  | Not Present. |  |  |
| Packet flow Identifier |  | Not Present. |  |  |
|  |  | '0000 0000'B | Best Effort | pc_GERAN |
| APN-AMBR |  | Not Present. |  |  |
| Protocol configuration options |  | Not present |  |  |
| WLAN offload container |  | Not present |  |  |
| NBIFOM container |  | Not present |  |  |
| Header compression configuration |  | Not present |  |  |
| Extended protocol configuration options |  | Not present |  |  |

| Condition | Explanation |
| --- | --- |
| NETWORK-INITIATED | Network initiated ESM procedures |
| UE-INITIATED | UE initiated ESM procedures |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

#### -	NOTIFICATION

This message is sent by the SS to the UE.

Table 4.7.3-18A: NOTIFICATION

| Derivation Path: 24.301 clause 8.3.18A |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | ESM |  |  |
| EPS bearer identity |  | '0000'B | No EPS bearer identity assigned |  |
|  |  | The same value as the value set in ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message for IMS signalling. |  | SRVCC-HO-CANCELLED |
| Procedure transaction identity |  | '0000 0000'B | No procedure transaction identity assigned |  |
| Notification message identity |  | '1101 1011'B | Notification |  |
| Notification indicator |  | '0000 0001'B | SRVCC handover cancelled, IMS session re-establishment required | SRVCC-HO-CANCELLED |

| Condition | Explanation |
| --- | --- |
| SRVCC-HO-CANCELLED | Used if SRVCC handover is cancelled. |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

#### -	PDN CONNECTIVITY REJECT

This message is sent by the SS to the UE.

Table 4.7.3-19: PDN CONNECTIVITY REJECT

| Derivation Path: 24.301 clause 8.3.17 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | ESM |  |  |
| EPS bearer identity |  | '0000'B | No EPS bearer identity assigned |  |
| Procedure transaction identity |  | The same value as the value set in PDN CONNECTIVITY REQUEST message. |  |  |
| PDN connectivity reject message identity |  | '1101 0001'B | PDN connectivity reject |  |
| ESM cause |  | The value is set according to specific message content. |  |  |
| Protocol configuration options |  | Not present |  |  |
| NBIFOM container |  | Not present |  |  |
| Extended protocol configuration options |  | Not present |  |  |

NOTE:	This message is sent without integrity protection before NAS security mode control procedure has been successfully completed and sent within SECURITY PROTECTED NAS MESSAGE message after NAS security mode control procedure has been successfully completed.

#### -	PDN CONNECTIVITY REQUEST

This message is sent by the UE to the SS.

Table 4.7.3-20: PDN CONNECTIVITY REQUEST

| Derivation Path: 24.301 clause 8.3.20 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | ESM |  |  |
| EPS bearer identity |  | '0000'B | No EPS bearer identity assigned |  |
| Procedure transaction identity |  | Any value from 1 to 254 |  |  |
| PDN connectivity request message identity |  | '1101 0000'B | PDN connectivity request |  |
| Request type |  | '0001'B | initial request |  |
| PDN type |  | Any value between '001'B, '010'B, '011'B and '100'B | The allowed values are respectively IPv4, IPv6, IPv4v6 and "unused but interpreted as IPv6 by the network" |  |
| PDN type |  | ‘101’B | Non-IP | CIoT_CP_NON_IP |
| ESM information transfer flag |  | Not present or any allowed value |  |  |
| Access point name |  | Not present | The value is mandatory when the PDN CONNECTIVITY REQUEST message is sent together with an ATTACH REQUEST message. |  |
| Protocol configuration options |  | Not present or any allowed value | The value received from the UE does not affect the possible verdict associated with the message when received by the SS. The SS shall remember if this IE is present and its contents because this affects subsequent SS behaviour, e.g. coding of ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST. | NOT (pc_NB or pc_NonIP_PDN) |
| Protocol configuration options |  | Not present |  | pc_NB or pc_NonIP_PDN |
| Protocol configuration options |  | If present including at least the following container |  | Interworking_with_5GS |
| Container ID n |  | ‘001A’H | n assigned to next available number |  |
| Length of container ID n contents |  |  | 1 octet |  |
| Container ID n contents |  | PDU session ID | PDU session ID assigned by the UE |  |
| NBIFOM container |  | Not present or any allowed value |  |  |
| Header compression configuration |  | Not present or any allowed value |  |  |
| Extended protocol configuration options |  | Not present or any allowed value |  |  |
| Extended protocol configuration options |  | If present including at least the following container |  | Interworking_with_5GS |
| Container ID n |  | ‘001A’H | n assigned to next available number |  |
| Length of container ID n contents |  |  | 1 octet |  |
| Container ID n contents |  | PDU session ID | PDU session ID assigned by the UE |  |

| Condition | Explanation |
| --- | --- |
| CIoT_CP_NON_IP | If the UE requests a Non-IP type PDN |
| Interworking_with_5GS | If the UE has indicated support of N1, then the SS shall include this IE to provide details for the interworking with 5GS. Applicable to 5G test cases defined in TS 38.523-1 [71]. Note: The PDU session ID container shall be sent in either the Protocol configuration options or the Extended protocol configuration options IE. |

NOTE:	This message is sent without integrity protection before NAS security mode control procedure has been successfully completed and sent within SECURITY PROTECTED NAS MESSAGE message after NAS security mode control procedure has been successfully completed.

#### -	PDN DISCONNECT REJECT

This message is sent by the SS to the UE.

Table 4.7.3-21: PDN DISCONNECT REJECT

| Derivation Path: 24.301 clause 8.3.19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | ESM |  |  |
| EPS bearer identity |  | '0000'B | No EPS bearer identity assigned |  |
| Procedure transaction identity |  | The value indicated in PDN DISCONNECT REQUEST message. |  |  |
| PDN disconnect reject message identity |  | '1101 0011'B | PDN disconnect reject |  |
| ESM cause |  | Set according to specific message content. |  |  |
| Protocol configuration options |  | Not present |  |  |
| Extended protocol configuration options |  | Not present |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

#### -	PDN DISCONNECT REQUEST

This message is sent by the UE to the SS.

Table 4.7.3-22: PDN DISCONNECT REQUEST

| Derivation Path: 24.301 clause 8.3.20 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | ESM |  |  |
| EPS bearer identity |  | '0000'B | No EPS bearer identity assigned |  |
| Procedure transaction identity |  | Any value from 1 to 254 |  |  |
| PDN disconnect request message identity |  | '1101 0010'B | PDN disconnect request |  |
| Linked EPS bearer identity |  | The EPS bearer identity of the associated default bearer. |  |  |
| Spare half octet |  | '0000'B |  |  |
| Protocol configuration options |  | Not present or any allowed value |  |  |
| Extended protocol configuration options |  | Not present or any allowed value |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

## 4.7A	Default TC message and information element contents

This clause contains the default values of common TC (Test Control, see [38]) messages and information elements, which apply to all test cases unless otherwise specified. All the messages and information elements are listed in alphabetical order.

#### -	ACTIVATE TEST MODE

This message is sent by the SS to the UE embedded in a RRC DLInformationTransfer message.

Table 4.7A-1: ACTIVATE TEST MODE

| Derivation Path: 36.509 clause 6.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | 1 1 1 1 |  |  |
| Skip indicator |  | 0 0 0 0 |  |  |
| Message type |  | 1 0 0 0 0 1 0 0 |  |  |
| UE test loop mode |  | 0 0 0 0 0 0 0 0 | UE test loop mode A | UE TEST LOOP MODE A (default) |
| UE test loop mode |  | 0 0 0 0 0 0 0 1 | UE test loop mode B | UE TEST LOOP MODE B |
| UE test loop mode |  | 0 0 0 0 0 0 1 0 | UE test loop mode C | UE TEST LOOP MODE C |
| UE test loop mode |  | 0 0 0 0 0 011 | UE test loop mode D | UE TEST LOOP MODE D |
| UE test loop mode |  | 0 0 0 0 0 1 0 0 | UE test loop mode E | UE TEST LOOP MODE E |
| UE test loop mode |  | 0 0 0 0 0 1 0 1 | UE test loop mode F | UE TEST LOOP MODE F |

| Condition | Explanation |
| --- | --- |
| UE TEST LOOP MODE A | UE test loop function configured for UE test loop mode A operation. |
| UE TEST LOOP MODE B | UE test loop function configured for UE test loop mode B operation. |
| UE TEST LOOP MODE C | UE test loop function configured for UE test loop mode C operation. |
| UE TEST LOOP MODE D | UE test loop function configured for UE test loop mode D operation. |
| UE TEST LOOP MODE E | UE test loop function configured for UE test loop mode E operation. |
| UE TEST LOOP MODE F | UE test loop function configured for UE test loop mode F operation. |

#### -	ACTIVATE TEST MODE COMPLETE

This message is sent by the UE to the SS embedded in a RRC ULInformationTransfer message.

Table 4.7A-2: ACTIVATE TEST MODE COMPLETE

| Derivation Path: 36.509 clause 6.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | 1 1 1 1 |  |  |
| Skip indicator |  | 0 0 0 0 |  |  |
| Message type |  | 1 0 0 0 0 1 0 1 |  |  |

#### -	CLOSE UE TEST LOOP

This message is sent by the SS to the UE embedded in a RRC DLInformationTransfer message.

Table 4.7A-3: CLOSE UE TEST LOOP

| Derivation Path: 36.509 clause 6.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | 1 1 1 1 |  |  |
| Skip indicator |  | 0 0 0 0 |  |  |
| Message type |  | 1 0 0 0 0 0 0 0 |  |  |
| UE test loop mode |  | 0 0 0 0 0 0 0 0 | UE test loop mode A | UE TEST LOOP MODE A (default) |
| UE test loop mode A LB setup |  |  |  |  |
| Length of UE test loop mode A LB setup list in bytes |  | 0 | No LB setup list. No scaling (UL RLC SDU size will be equal to the received DL SDU size) |  |
| UE test loop mode B LB setup |  | Not present |  |  |
| UE test loop mode C LB setup |  | Not present |  |  |
| UE test loop mode D LB setup |  | Not present |  |  |
| UE test loop mode E LB setup |  | Not present |  |  |
| UE test loop mode F setup |  | Not present |  |  |
| UE test loop mode |  | 0 0 0 0 0 0 0 1 | UE test loop mode B | UE TEST LOOP MODE B |
| UE test loop mode A LB setup |  | Not present |  |  |
| UE test loop mode B LB setup |  |  |  |  |
| IP PDU delay |  | 0 0 0 0 0 0 0 0 | No delay |  |
| UE test loop mode C LB setup |  | Not present |  |  |
| UE test loop mode D LB setup |  | Not present |  |  |
| UE test loop mode E LB setup |  | Not present |  |  |
| UE test loop mode F setup |  | Not present |  |  |
| UE test loop mode |  | 0 0 0 0 0 0 1 0 | UE test loop mode C | UE TEST LOOP MODE C |
| UE test loop mode A LB setup |  | Not present |  |  |
| UE test loop mode B LB setup |  | Not present |  |  |
| UE test loop mode C LB setup |  |  | MTCH ID |  |
| MBSFN area identity |  | 0 0 0 0 0 0 0 0 | 0, same value as broadcasted in the default SystemInformationBlockType13 message |  |
| MCH identity |  | 0 0 0 0 0 0 0 0 | 0, same value as indicated in the default MBSFNAreaConfiguration message |  |
| Logical channel identity |  | 0 0 0 0 0 0 0 1 | 1, same value as indicated in the default MBSFNAreaConfiguration message |  |
| UE test loop mode D LB setup |  | Not present |  |  |
| UE test loop mode E LB setup |  | Not present |  |  |
| UE test loop mode F setup |  | Not present |  |  |
| UE test loop mode |  | 0 0 0 0 0 0 1 1 | UE test loop mode D | UE TEST LOOP MODE D |
| UE test loop mode A LB setup |  | Not present |  |  |
| UE test loop mode B LB setup |  | Not present |  |  |
| UE test loop mode C LB setup |  | Not present |  |  |
| UE test loop mode D LB setup |  |  |  |  |
| Discovery Announce or Monitor |  | 0 0 0 0 0 0 0 0 | 0, triggered to monitor sidelink direct discovery by default. |  |
| Length of UE test loop mode D Monitor list in bytes |  | 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 | 0 byte because by default no specific App code is monitored individually. The UE will collectively monitor all App codes. |  |
| UE test loop mode E LB setup |  | Not present |  |  |
| UE test loop mode F setup |  | Not present |  |  |
| UE test loop mode |  | 0 0 0 0 0 1 0 0 | UE test loop mode E | UE TEST LOOP MODE E |
| UE test loop mode A LB setup |  | Not present |  |  |
| UE test loop mode B LB setup |  | Not present |  |  |
| UE test loop mode C LB setup |  | Not present |  |  |
| UE test loop mode D LB setup |  | Not present |  |  |
| UE test loop mode E LB setup |  |  |  |  |
| Length of UE test loop mode E Monitor setup contents in bytes |  | 0 0 0 0 0 0 0 1 | 1 to account for first 1 octet which is always present. By default no specific Group Destination ID code is monitored individually. The UE will collectively monitor packets for all Group Destination IDs. |  |
| Communication Transmit or Receive |  | 0 0 0 0 0 0 0 0 | 0, triggered to receive sidelink direct communication by default. |  |
| UE test loop mode F setup |  | Not present |  |  |
| UE test loop mode |  | 0 0 0 0 0 1 0 0 | UE test loop mode E | UE TEST LOOP MODE E (V2X Reception) |
| UE test loop mode A LB setup |  | Not present |  |  |
| UE test loop mode B LB setup |  | Not present |  |  |
| UE test loop mode C LB setup |  | Not present |  |  |
| UE test loop mode D LB setup |  | Not present |  |  |
| UE test loop mode E LB setup |  |  |  |  |
| Length of UE test loop mode E Monitor setup contents in bytes |  | 0 0 0 0 0 0 0 1 | 1 to account for first 1 octet which is always present. By default no specific Group Destination ID code is monitored individually. The UE will collectively monitor packets for all Group Destination IDs. |  |
| Communication Transmit or Receive |  | 0 0 0 0 0 0 1 0 | ‘10’ indicates V2X UE triggered to receive V2X communication by default. |  |
| UE test loop mode F setup |  | Not present |  |  |
| UE test loop mode |  | 0 0 0 0 0 1 0 0 | UE test loop mode E | UE TEST LOOP MODE E(V2X Transmission) |
| UE test loop mode A LB setup |  | Not present |  |  |
| UE test loop mode B LB setup |  | Not present |  |  |
| UE test loop mode C LB setup |  | Not present |  |  |
| UE test loop mode D LB setup |  | Not present |  |  |
| UE test loop mode E LB setup |  |  |  |  |
| Length of UE test loop mode E Monitor setup contents in bytes |  | 0 0 0 0 0 0 0 1 | 1 to account for first 1 octet which is always present. By default no specific Group Destination ID code is monitored individually. The UE will collectively monitor packets for all Group Destination IDs. |  |
| Communication Transmit or Receive |  | 0 0 0 0 0 0 1 1 | ‘11’ indicates V2X UE triggered to transmit V2Xcommunication by default. |  |
| UE test loop mode F setup |  | Not present |  |  |
| UE test loop mode |  | 0 0 0 0 0 1 0 1 | UE test loop mode F | UE TEST LOOP MODE F |
| UE test loop mode A LB setup |  | Not present |  |  |
| UE test loop mode B LB setup |  | Not present |  |  |
| UE test loop mode C LB setup |  | Not present |  |  |
| UE test loop mode D LB setup |  | Not present |  |  |
| UE test loop mode E LB setup |  | Not present |  |  |
| UE test loop mode F setup |  |  | SC-MTCH ID |  |
| g-RNTI |  | ‘0100’H | same value as indicated in the default SCPTM-Configuration message |  |

| Condition | Explanation |
| --- | --- |
| UE TEST LOOP MODE A | UE test loop function configured for UE test loop mode A operation. |
| UE TEST LOOP MODE B | UE test loop function configured for UE test loop mode B operation. |
| UE TEST LOOP MODE C | UE test loop function configured for UE test loop mode C operation. |
| UE TEST LOOP MODE D | UE test loop function configured for UE test loop mode D operation. |
| UE TEST LOOP MODE E | UE test loop function configured for UE test loop mode E operation. |
| UE TEST LOOP MODE E (V2X Reception) | UE test loop function configured for UE test loop mode E operation with V2X reception. |
| UE TEST LOOP MODE E(V2X Transmission) | UE test loop function configured for UE test loop mode E operation with V2X transmission. |
| UE TEST LOOP MODE F | UE test loop function configured for UE test loop mode F operation. |

#### -	CLOSE UE TEST LOOP COMPLETE

This message is sent by the UE to the SS embedded in a RRC ULInformationTransfer message.

Table 4.7A-4: CLOSE UE TEST LOOP

| Derivation Path: 36.509 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | 1 1 1 1 |  |  |
| Skip indicator |  | 0 0 0 0 |  |  |
| Message type |  | 1 0 0 0 0 0 0 1 |  |  |

#### -	DEACTIVATE TEST MODE

This message is sent by the SS to the UE embedded in a RRC DLInformationTransfer message.

Table 4.7A-5: DEACTIVATE TEST MODE

| Derivation Path: 36.509 clause 6.7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | 1 1 1 1 |  |  |
| Skip indicator |  | 0 0 0 0 |  |  |
| Message type |  | 1 0 0 0 0 1 1 0 |  |  |

#### -	DEACTIVATE TEST MODE COMPLETE

This message is sent by the UE to the SS embedded in a RRC ULInformationTransfer message.

Table 4.7A-6: DEACTIVATE TEST MODE COMPLETE

| Derivation Path: 36.509 clause 6.8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | 1 1 1 1 |  |  |
| Skip indicator |  | 0 0 0 0 |  |  |
| Message type |  | 1 0 0 0 0 1 1 1 |  |  |

#### -	OPEN UE TEST LOOP

This message is sent by the SS to the UE embedded in a RRC DLInformationTransfer message.

Table 4.7A-7: OPEN UE TEST LOOP

| Derivation Path: 36.509 clause 6.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | 1 1 1 1 |  |  |
| Skip indicator |  | 0 0 0 0 |  |  |
| Message type |  | 1 0 0 0 0 0 1 0 |  |  |

#### -	OPEN UE TEST LOOP COMPLETE

This message is sent by the UE to the SS embedded in a RRC ULInformationTransfer message.

Table 4.7A-8: OPEN UE TEST LOOP COMPLETE

| Derivation Path: 36.509 clause 6.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | 1 1 1 1 |  |  |
| Skip indicator |  | 0 0 0 0 |  |  |
| Message type |  | 1 0 0 0 0 0 1 1 |  |  |

#### -	UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST

This message is sent by the SS to the UE embedded in a RRC DLInformationTransfer message.

Table 4.7A-9: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST

| Derivation Path: 36.509 clause 6.10 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | 1 1 1 1 |  |  |
| Skip indicator |  | 0 0 0 0 |  |  |
| Message type |  | 1 0 0 0 1 0 0 1 |  |  |

#### -	UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE

This message is sent by the UE to the SS embedded in a RRC ULInformationTransfer message.

Table 4.7A-10: UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE

| Derivation Path: 36.509 clause 6.11 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | 1 1 1 1 |  |  |
| Skip indicator |  | 0 0 0 0 |  |  |
| Message type |  | 1 0 0 0 1 0 1 1 |  |  |
| MBMS Packet Counter Value |  |  |  |  |

#### -	UE TEST LOOP PROSE PACKET COUNTER REQUEST

This message is sent by the SS to the UE embedded in a RRC DLInformationTransfer message.

Table 4.7A-11: UE TEST LOOP PROSE PACKET COUNTER REQUEST

| Derivation Path: 36.509 clause 6.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | 1 1 1 1 |  |  |
| Skip indicator |  | 0 0 0 0 |  |  |
| Message type |  | 1 0 0 0 1 1 0 0 |  |  |

#### -	UE TEST LOOP PROSE PACKET COUNTER RESPONSE

This message is sent by the UE to the SS embedded in a RRC ULInformationTransfer message.

Table 4.7A-12: UE TEST LOOP PROSE PACKET COUNTER RESPONSE

| Derivation Path: 36.509 clause 6.14 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | 1 1 1 1 |  |  |
| Skip indicator |  | 0 0 0 0 |  |  |
| Message type |  | 1 0 0 0 1 1 0 1 |  |  |
| ProSe Direct Discovery Packet Counter(s)Value |  |  |  | Mode D |
| ProSe Direct Communication PSCCH Packet Counter(s) Value |  |  |  | Mode E |
| ProSe Direct Communication STCH Packet Counter(s) Value |  |  |  | Mode E |

| Condition | Explanation |
| --- | --- |
| Mode D | This IE is present if UE test loop function is configured for UE test loop mode D operation else it shall be absent. |
| Mode E | This IE is present if UE test loop function is configured for UE test loop mode E operation else it shall be absent |

#### -	UE TEST LOOP MODE F SCPTM PACKET COUNTER REQUEST

This message is sent by the SS to the UE embedded in a RRC DLInformationTransfer message.

Table 4.7A-13: UE TEST LOOP MODE F SCPTM PACKET COUNTER REQUEST

| Derivation Path: 36.509 clause 6.15 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | 1 1 1 1 |  |  |
| Skip indicator |  | 0 0 0 0 |  |  |
| Message type |  | 1 0 0 0 1 1 1 0 |  |  |

#### -	UE TEST LOOP MODE F SCPTM PACKET COUNTER RESPONSE

This message is sent by the UE to the SS embedded in a RRC ULInformationTransfer message.

Table 4.7A-14: UE TEST LOOP MODE F SCPTM PACKET COUNTER RESPONSE

| Derivation Path: 36.509 clause 6.16 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | 1 1 1 1 |  |  |
| Skip indicator |  | 0 0 0 0 |  |  |
| Message type |  | 1 0 0 0 1 1 1 1 |  |  |
| SCPTM Packet Counter Value |  |  |  |  |

## 4.7B	Default UTRA message and information element contents

### 4.7B.1	UTRA RRC messages

#### –	HANDOVER TO UTRAN COMMAND

Table 4.7B.1-1: HANDOVER TO UTRAN COMMAND

| Derivation Path: 25.331, clause 10.2.16a |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| New U-RNTI | U-RNTI-Short |  |  |
| - SRNC identity | 0000 0000 0001B |  |  |
| - S-RNTI-2 | 00 0000 0001B |  |  |
| Ciphering algorithm | UEA0 or UEA1. The indicated algorithm must be one of the algorithms supported by the UE. |  |  |
| RNC support for change of UE capability | FALSE |  |  |
| New H-RNTI | Not present |  | UTRA FDD PS RB, UTRA TDD PS RB, UTRA Speech, UTRA PS RB + Speech, UTRA Speech + Packet RAB Setup after Speech RAB Setup in CELL_DCH |
|  | '1010 1010 1010 1010' |  | UTRA HSDPA RB, UTRA HSUPA/HSDPA RB |
| New Primary E-RNTI | Not present |  | UTRA FDD PS RB, UTRA TDD PS RB, UTRA HSDPA RB, UTRA Speech, UTRA PS RB + Speech. ,UTRA Speech + Packet RAB Setup after Speech RAB Setup in CELL_DCH |
|  | '1010 1010 1010 1010' |  | UTRA HSUPA/HSDPA RB |
| New Secondary E-RNTI | Not present |  |  |
| Default configuration for CELL_FACH | Not present |  |  |
| CHOICE specification mode | Complete specification |  |  |
| - Signalling RB information to setup List | Same as the corresponding IE in the RRC CONNECTION SETUP message (Transition to CELL_DCH or HS-PDSCH in CELL_FACH) using condition A1 in TS 34.108 clause 9.1.1 for UTRA-FDD. Same as the corresponding IE in the RRC CONNECTION SETUP message in TS 34.108 clause 9.1.2 for UTRA-TDD. |  | UTRA FDD PS RB, UTRA HSDPA RB, UTRA Speech, UTRA PS RB + Speech, ,UTRA Speech + Packet RAB Setup after Speech RAB Setup in CELL_DCH |
|  | Same as the corresponding IE in the RRC CONNECTION SETUP message in TS 34.108 clause 9.2.2 |  | UTRA TDD PS RB |
|  | Same as the corresponding IE in the RRC CONNECTION SETUP message (Transition to CELL_DCH or HS-PDSCH in CELL_FACH) using condition A3 in TS 34.108 clause 9.1.1 for UTRA-FDD. Same as the corresponding IE in the RRC CONNECTION SETUP (Transition to CELL_DCH) (1.28 Mcps TDD option) message in TS 34.108 clause 9.1.2 for UTRA-TDD. |  | UTRA HSUPA/HSDPA RB |
| - RAB information to setup list | Same as the corresponding IE in the RADIO BEARER SETUP message using condition A2 in TS 34.108 clause 9.1.1 for UTRA-FDD. Same as the corresponding IE in the RADIO BEARER SETUP message (1.28 Mcps TDD) using condition A2 in TS 34.108 clause 9.1.2 for UTRA-TDD. |  | UTRA Speech, UTRA PS RB + Speech, UTRA Speech + Packet RAB Setup after Speech RAB Setup in CELL_DCH |
|  | Same as the corresponding IE in the RADIO BEARER SETUP message using condition A3 in TS 34.108 clause 9.1.1 for UTRA-FDD. Same as the corresponding IE in the RADIO BEARER SETUP message (1.28 Mcps TDD) using condition A3 in TS 34.108 clause 9.1.2 for UTRA-TDD. |  | UTRA FDD PS RB, UTRA PS RB + Speech |
|  | Same as the corresponding IE in the RADIO BEARER SETUP message using condition A11 in TS 34.108 clause 9.1.1 for UTRA-FDD. Same as the corresponding IE in the RADIO BEARER SETUP message (1.28 Mcps TDD) using condition A11 in TS 34.108 clause 9.1.2 for UTRA-TDD. |  | UTRA Speech + Packet RAB Setup after Speech RAB Setup in CELL_DCH |
|  | Same as the corresponding IE in the RADIO BEARER SETUP message (1.28 Mcps TDD) using condition A3 in TS 34.108 clause 9.2.2 for UTRA-TDD. |  | UTRA TDD PS RB |
|  | Same as the corresponding IE in the RADIO BEARER SETUP message using condition A10 in TS 34.108 clause 9.1.1 for UTRA-FDD. Same as the corresponding IE in the RADIO BEARER SETUP message (1.28 Mcps TDD) using condition A10 in TS 34.108 clause 9.1.2 for UTRA-TDD. |  | UTRA HSDPA RB |
|  | Same as the corresponding IE in the RADIO BEARER SETUP message using condition A14 in TS 34.108 clause 9.1.1 for UTRA-FDD. Same as the corresponding IE in the RADIO BEARER SETUP message (1.28 Mcps TDD) using condition A13 in TS 34.108 clause 9.1.2 for UTRA-TDD. |  | UTRA HSUPA/HSDPA RB |
| - UL Transport channel information common for all transport channels | Same as the corresponding IE in the RADIO BEARER SETUP message using condition A2 in TS 34.108 clause 9.1.1 for UTRA-FDD Same as the corresponding IE in the RADIO BEARER SETUP message (1.28 Mcps TDD) using condition A2 in TS 34.108 clause 9.1.2 for UTRA-TDD |  | UTRA Speech, UTRA PS RB + Speech, UTRA Speech + Packet RAB Setup after Speech RAB Setup in CELL_DCH |
|  | Same as the corresponding IE in the RADIO BEARER SETUP message using condition A3 in TS 34.108 clause 9.1.1 UTRA-FDD. Same as the corresponding IE in the RADIO BEARER SETUP message (1.28 Mcps TDD) using condition A3 in TS 34.108 clause 9.2.2 UTRA-TDD. |  | UTRA FDD PS RB, UTRA TDD PS RB |
|  | Same as the corresponding IE in the RADIO BEARER SETUP message using condition A10 in TS 34.108 clause 9.1.1 for UTRA-FDD. Same as the corresponding IE in the RADIO BEARER SETUP message (1.28 Mcps TDD) using condition A10 in TS 34.108 clause 9.1.2 for UTRA-TDD. |  | UTRA HSDPA RB |
|  | Same content as the corresponding IE in the RADIO BEARER SETUP message using condition A14 in TS 34.108 clause 9.1.1 for UTRA-FDD. Same content as the corresponding IE in the RADIO BEARER SETUP message (1.28 Mcps TDD) using condition A13 in TS 34.108 clause 9.1.2 for UTRA-TDD. |  | UTRA HSUPA/HSDPA RB |
| - Added or Reconfigured UL TrCH information | Same as the corresponding IE in the RADIO BEARER SETUP message using condition A2 in TS 34.108 clause 9.1.1 for UTRA-FDD. Same as the corresponding IE in the RADIO BEARER SETUP message (1.28 Mcps TDD) using condition A2 in TS 34.108 clause 9.1.2 for UTRA-TDD |  | UTRA Speech, UTRA PS RB + Speech, UTRA Speech + Packet RAB Setup after Speech RAB Setup in CELL_DCH |
|  | Same as the corresponding IE in the RADIO BEARER SETUP message using condition A3 in TS 34.108 clause 9.1.1 |  | UTRA FDD PS RB |
|  | Same as the corresponding IE in the RADIO BEARER SETUP message using condition A10 in TS 34.108 clause 9.1.1 for UTRA-FDD. Same as the corresponding IE in the RADIO BEARER SETUP message (1.28 Mcps TDD) using condition A10 in TS 34.108 clause 9.1.2 for UTRA-TDD. |  | UTRA HSDPA RB |
|  | Same as the corresponding IE in the RADIO BEARER SETUP message using condition A11 in TS 34.108 clause 9.1.1 for UTRA-FDD. Same as the corresponding IE in the RADIO BEARER SETUP message (1.28 Mcps TDD) using condition A11 in TS 34.108 clause 9.1.2 for UTRA-TDD. |  | UTRA Speech + Packet RAB Setup after Speech RAB Setup in CELL_DCH |
|  | Same as the corresponding IE in the RADIO BEARER SETUP message using condition A14 in TS 34.108 clause 9.1.1 for UTRA-FDD. Same as the corresponding IE in the RADIO BEARER SETUP message (1.28 Mcps TDD) using condition A13 in TS 34.108 clause 9.1.2 for UTRA-TDD. |  | UTRA HSUPA/HSDPA RB |
| - Added or Reconfigured UL TrCH information | 2 Entries |  | UTRA TDD PS RB |
| - Added or Reconfigured UL TrCH information[1] | Same as the corresponding IE in the RRC CONNECTION SETUP message (1.28 Mcps TDD) in TS 34.108 clause 9.2.2. |  | UTRA TDD PS RB |
| - Added or Reconfigured UL TrCH information[2] | Same as the corresponding IE in the RADIO BEARER SETUP message (1.28 Mcps TDD) using condition A3 in TS 34.108 clause 9.2.2. |  | UTRA TDD PS RB |
| - DL Transport channel information common for all transport channels | Same as the corresponding IE in the RADIO BEARER SETUP message using condition A2 in TS 34.108 clause 9.1.1 for UTRA-FDD. Same as the corresponding IE in the RADIO BEARER SETUP message (1.28 Mcps TDD) using condition A2 in TS 34.108 clause 9.1.2 for UTRA-TDD. |  | UTRA Speech, UTRA PS RB + Speech, UTRA Speech + Packet RAB Setup after Speech RAB Setup in CELL_DCH |
|  | Same as the corresponding IE in the RADIO BEARER SETUP message using condition A3 in TS 34.108 clause 9.1.1 |  | UTRA FDD PS RB |
|  | Same as the corresponding IE in the RADIO BEARER SETUP message (1.28 Mcps TDD) using condition A3 in TS 34.108 clause 9.2.2. |  | UTRA TDD PS RB |
|  | Same as the corresponding IE in the RADIO BEARER SETUP message using condition A10 in TS 34.108 clause 9.1.1 for UTRA-FDD. Same as the corresponding IE in the RADIO BEARER SETUP message (1.28 Mcps TDD) using condition A10 in TS 34.108 clause 9.1.2 for UTRAT-TDD. |  | UTRA HSDPA RB |
|  | Same content as the corresponding IE in the RADIO BEARER SETUP message using condition A14 in TS 34.108 clause 9.1.1 for UTRA-FDD. Same as the corresponding IE in the RADIO BEARER SETUP message (1.28 Mcps TDD) using condition A13 in TS 34.108 clause 9.1.2 for UTRA-TDD. |  | UTRA HSUPA/HSDPA RB |
| - Added or Reconfigured DL TrCH information | Same as the corresponding IE in the RADIO BEARER SETUP message using condition A2 in TS 34.108 clause 9.1.1 for UTRA-FDD. Same as the corresponding IE in the RADIO BEARER SETUP message (1.28 Mcps TDD) using condition A2 in TS 34.108 clause 9.1.2 for UTRA-TDD. |  | UTRA Speech, UTRA PS RB + Speech, UTRA Speech + Packet RAB Setup after Speech RAB Setup in CELL_DCH |
|  | Same as the corresponding IE in the RADIO BEARER SETUP message using condition A3 in TS 34.108 clause 9.1.1 |  | UTRA FDD PS RB |
|  | Same as the corresponding IE in the RADIO BEARER SETUP message using condition A11 in TS 34.108 clause 9.1.1 for UTRA-FDD. Same as the corresponding IE in the RADIO BEARER SETUP message (1.28 Mcps TDD) using condition A11 in TS 34.108 clause 9.1.2 for UTRA-TDD. |  | UTRA Speech + Packet RAB Setup after Speech RAB Setup in CELL_DCH |
|  | Same as the corresponding IE in the RADIO BEARER SETUP message using condition A10 in TS 34.108 clause 9.1.1 for UTRA-FDD. Same as the corresponding IE in the RADIO BEARER SETUP message (1.28 Mcps TDD) using condition A10 in TS 34.108 clause 9.1.2 for UTRA-TDD. |  | UTRA HSDPA RB |
|  | Same as the corresponding IE in the RADIO BEARER SETUP message using condition A14 in TS 34.108 clause 9.1.1 for UTRA-FDD. Same as the corresponding IE in the RADIO BEARER SETUP message (1.28 Mcps TDD) using condition A13 in TS 34.108 clause 9.1.2 for UTRA-TDD. |  | UTRA HSUPA/HSDPA RB |
| - Added or Reconfigured DL TrCH information | 2 Entries |  | UTRA TDD PS RB |
| - Added or Reconfigured DL TrCH information[1] | Same as the corresponding IE in the RRC CONNECTION SETUP message (1.28 Mcps TDD) in TS 34.108 clause 9.2.2. |  | UTRA TDD PS RB |
| - Added or Reconfigured DL TrCH information[2] | Same as the corresponding IE in the RADIO BEARER SETUP message (1.28 Mcps TDD) using condition A3 in TS 34.108 clause 9.2.2. |  | UTRA TDD PS RB |
| - Uplink DPCH info | Same as the corresponding IE in the RADIO BEARER SETUP message using condition A2 in TS 34.108 clause 9.1.1 for UTRA-FDD. Same as the corresponding IE in the RADIO BEARER SETUP message (1.28 Mcps TDD) using condition A13 in TS 34.108 clause 9.1.2 for UTRA-TDD. |  | UTRA Speech, UTRA PS RB + Speech, UTRA Speech + Packet RAB Setup after Speech RAB Setup in CELL_DCH |
|  | Same as the corresponding IE in the RADIO BEARER SETUP message using condition A3 in TS 34.108 clause 9.1.1 |  | UTRA FDD PS RB |
|  | Same as the corresponding IE in the RADIO BEARER SETUP message (1.28 Mcps TDD) using condition A3 in TS 34.108 clause 9.2.2. |  | UTRA TDD PS RB |
|  | Same as the corresponding IE in the RADIO BEARER SETUP message using condition A10 in TS 34.108 clause 9.1.1 for UTRA-FDD. Same as the corresponding IE in the RADIO BEARER SETUP message (1.28 Mcps TDD) using condition A10 in TS 34.108 clause 9.1.2 for UTRA-TDD. |  | UTRA HSDPA RB |
|  | Same as the corresponding IE in the RADIO BEARER SETUP message using condition A14 in TS 34.108 clause 9.1.1 for UTRA-FDD. Same as the corresponding IE in the RADIO BEARER SETUP message (1.28 Mcps TDD) using condition A13 in TS 34.108 clause 9.1.2 for UTRA-TDD. |  | UTRA HSUPA/HSDPA RB |
| - E-DCH Info | Not present |  | UTRA FDD PS RB, UTRA TDD PS RB, UTRA HSDPA RB, UTRA Speech, UTRA PS RB + Speech, UTRA Speech + Packet RAB Setup after Speech RAB Setup in CELL_DCH |
|  | Same as the corresponding IE in the RADIO BEARER SETUP message using condition A14 in TS 34.108 clause 9.1.1 for UTRA-FDD. Same as the corresponding IE in the RADIO BEARER SETUP message (1.28 Mcps TDD) using condition A13 in TS 34.108 clause 9.1.2 for UTRA-TDD. |  | UTRA HSUPA/HSDPA RB |
| - Downlink HS-PDSCH Information | Not present |  | UTRA FDD PS RB, UTRA TDD PS RB, UTRA Speech, UTRA PS RB + Speech, UTRA Speech + Packet RAB Setup after Speech RAB Setup in CELL_DCH |
|  | Same as the corresponding IE in the RADIO BEARER SETUP message using condition A10 in TS 34.108 clause 9.1.1 for UTRA-FDD. Same as the corresponding IE in the RADIO BEARER SETUP message (1.28 Mcps TDD) using condition A10 in TS 34.108 clause 9.1.2 for UTRA-TDD. |  | UTRA HSDPA RB |
|  | Same as the corresponding IE in the RADIO BEARER SETUP message using condition A14 in TS 34.108 clause 9.1.1 for UTRA-FDD. Same as the corresponding IE in the RADIO BEARER SETUP message (1.28 Mcps TDD) using condition A13 in TS 34.108 clause 9.1.2 for UTRA-TDD. |  | UTRA HSUPA/HSDPA RB |
| - Downlink information common for all radio links | Same as the corresponding IE in the RADIO BEARER SETUP message using condition A2 in TS 34.108 clause 9.1.1 for UTRA-FDD. Same as the corresponding IE in the RADIO BEARER SETUP message (1.28 Mcps TDD) using condition A2 in TS 34.108 clause 9.1.2 for UTRA-TDD. |  | UTRA Speech, UTRA PS RB + Speech, UTRA Speech + Packet RAB Setup after Speech RAB Setup in CELL_DCH |
|  | Same as the corresponding IE in the RADIO BEARER SETUP message using condition A4 in TS 34.108 clause 9.1.1 |  | UTRA FDD PS RB |
|  | Same as the corresponding IE in the RADIO BEARER SETUP message (1.28 Mcps TDD) using condition A3 in TS 34.108 clause 9.2.2. |  | UTRA TDD PS RB |
|  | Same as the corresponding IE in the RADIO BEARER SETUP message using condition A10 in TS 34.108 clause 9.1.1 for UTRA-FDD. Same as the corresponding IE in the RADIO BEARER SETUP message (1.28 Mcps TDD) using condition A10 in TS 34.108 clause 9.1.2 for UTRA-TDD. |  | UTRA HSDPA RB |
|  | Same as the corresponding IE in the RADIO BEARER SETUP message using condition A14 in TS 34.108 clause 9.1.1 for UTRA-FDD. Same as the corresponding IE in the RADIO BEARER SETUP message (1.28 Mcps TDD) using condition A13 in TS 34.108 clause 9.1.2 for UTRA-TDD. |  | UTRA HSUPA/HSDPA RB |
| - Downlink information for each radio link list | Same as the corresponding IE in the RADIO BEARER SETUP message using condition A2 in TS 34.108 clause 9.1.1 for UTRA-FDD. Same as the corresponding IE in the RADIO BEARER SETUP message (1.28 Mcps TDD) using condition A2 in TS 34.108 clause 9.1.2 for UTRA-TDD. |  | UTRA Speech, UTRA PS RB + Speech, UTRA Speech + Packet RAB Setup after Speech RAB Setup in CELL_DCH |
|  | Same as the corresponding IE in the RADIO BEARER SETUP message using condition A3 in TS 34.108 clause 9.1.1 |  | UTRA FDD PS RB |
|  | Same as the corresponding IE in the RADIO BEARER SETUP message (1.28 Mcps TDD) using condition A3 in TS 34.108 clause 9.2.2. |  | UTRA TDD PS RB |
|  | Same as the corresponding IE in the RADIO BEARER SETUP message using condition A10 in TS 34.108 clause 9.1.1 for UTRA-FDD. Same as the corresponding IE in the RADIO BEARER SETUP message (1.28 Mcps TDD) using condition A10 in TS 34.108 clause 9.1.2 for UTRA-TDD. |  | UTRA HSDPA RB |
|  | Same as the corresponding IE in the RADIO BEARER SETUP message using condition A14 in TS 34.108 clause 9.1.1 for UTRA-FDD. Same as the corresponding IE in the RADIO BEARER SETUP message (1.28 Mcps TDD) using condition A13 in TS 34.108 clause 9.1.2 for UTRA-TDD. |  | UTRA HSUPA/HSDPA RB |
| Frequency info | Set according to the frequency of the target UTRA cell. |  |  |
| Multi-frequency Info | Not present |  |  |
| Maximum allowed UL TX power | 21dBm |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA Speech | Establishment of the UTRA reference configuration as specified in Table 4.8.3-1 using the condition “UTRA Speech”. |
| UTRA FDD PS RB | Establishment of the UTRA FDD reference configuration as specified in Table 4.8.3-1 using the condition “UTRA FDD PS RB”. |
| UTRA TDD PS RB | Establishment of the UTRA TDD reference configuration as specified in Table 4.8.3-1 using the condition “UTRA TDD PS RB”. |
| UTRA HSDPA RB | Establishment of the UTRA reference configuration as specified in Table 4.8.3-1 using the condition “UTRA HSDPA RB”. |
| UTRA HSUPA/HSDPA RB | Establishment of the UTRA reference configuration as specified in Table 4.8.3-1 using the condition “UTRA HSUPA/HSDPA RB”. |
| UTRA PS RB + Speech | Establishment of the UTRA reference configuration as specified in Table 4.8.3-1 using the condition “UTRA PS RB + Speech”. |
| UTRA Speech + Packet RAB Setup after Speech RAB Setup in CELL_DCH | Establishment of the UTRA reference configuration as specified in Table 4.8.3-1 using the condition “UTRA Speech + Packet RAB Setup after Speech RAB Setup in Cell DCH”. |

#### –	HANDOVER FROM UTRAN COMMAND

Table 4.7B.1-2: HANDOVER FROM UTRAN COMMAND

| Derivation Path: 25.331, clause 10.2.15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Message Type |  |  |  |
| Integrity check info |  |  |  |
| - Message authentication code | SS calculates the value of MAC-I for this message and writes to this IE. The first/ leftmost bit of the bit string contains the most significant bit of the MAC-I. |  |  |
| - RRC Message sequence number | SS provides the value of this IE, from its internal counter. |  |  |
| SR-VCC Info | Not present |  |  |
| Activation time | Now |  |  |
| RAB Info |  |  |  |
| - RAB identity | 0000 0101B The first/ leftmost bit of the bit string contains the most significant bit of the RAB identity. |  |  |
| - CN domain identity | PS domain |  |  |
| - NAS Synchronization Indicator | Not present |  |  |
| - Re-establishment timer | Use T315 |  |  |
| Inter-system message |  |  |  |
| - CHOICE System type | E-UTRA |  |  |
| - E-UTRA message | RRCConnectionReconfiguration using condition HO-TO-EUTRA(1,0) | See Table 4.6.1-8 |  |

#### –	MEASUREMENT CONTROL

Table 4.7B.1-3: MEASUREMENT CONTROL

| Derivation Path: 25.331, clause 10.2.17 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| Message Type |  |  |  |  |  |  |  |
| RRC transaction identifier |  | Arbitrarily selects an unused integer between 0 to 3 |  |  |  |  |  |
| Integrity check info |  |  |  |  |  |  |  |
| - Message authentication code |  | SS calculates the value of MAC-I for this message and writes to this IE. The first/ leftmost bit of the bit string contains the most significant bit of the MAC-I. |  |  |  |  |  |
| - RRC message sequence number |  | SS provides the value of this IE, from its internal counter. |  |  |  |  |  |
| Measurement Identity |  | 3 |  |  |  |  |  |
| Measurement Command |  | Setup |  |  |  |  |  |
| Measurement Reporting Mode |  |  |  |  |  |  |  |
| - Measurement Report Transfer Mode |  | Acknowledged mode RLC |  |  |  |  |  |
| - Periodical Reporting/Event Trigger Reporting Mode |  | Event trigger |  |  |  |  |  |
| Additional measurement list |  | Not present |  |  |  |  |  |
| CHOICE Measurement type |  | Inter-RAT measurement |  |  |  |  |  |
| - CHOICE Inter-RAT measurement objects |  | E-UTRA frequency list |  |  |  |  |  |
| - CHOICE E-UTRA frequency removal |  | Remove no frequencies |  |  |  |  |  |
| - New frequencies |  |  |  |  |  |  |  |
| - E-UTRA carrier frequency |  | Downlink EARFCN of E-UTRA Cell |  |  |  |  |  |
| - E-UTRA carrier frequency |  | 65535 |  |  |  | Band > 64 |  |
| - EARFCN extension |  | Downlink EARFCN of E-UTRA Cell |  |  |  | Band > 64 |  |
| - Measurement bandwidth |  | Same downlink system bandwidth as used for E-UTRA Cell |  |  |  |  |  |
| - Blacklisted cells list |  | Not present |  |  |  |  |  |
| - Inter-RAT measurement quantity |  |  |  |  |  |  |  |
| - Measurement quantity for UTRAN quality estimate |  |  |  |  |  |  |  |
| - Filter coefficient |  | 0 |  |  |  |  |  |
| - CHOICE mode |  | FDD |  |  |  | FDD_UTRA |  |
| - Measurement quantity |  | CPICH Ec/N0 |  |  |  |  |  |
| - Measurement quantity |  | CPICH RSCP |  | For signalling tests cases. |  |  |  |
| - CHOICE mode |  | TDD |  |  |  | TDD_UTRA |  |
| - Measurement quantity |  | Primary CCPCH RSCP |  |  |  |  |  |
| - CHOICE system |  | E-UTRA |  |  |  |  |  |
| - Measurement quantity |  | RSRP |  |  |  |  |  |
| - Filter coefficient |  | 0 |  |  |  |  |  |
| - Inter-RAT reporting quantity |  |  |  |  |  |  |  |
| - UTRAN estimated quality |  | FALSE |  |  |  |  |  |
| - CHOICE system |  | E-UTRA |  |  |  |  |  |
| - Reporting quantity |  | both |  |  |  |  |  |
| - Reporting cell status |  | Not present |  |  |  |  |  |
| - CHOICE report criteria |  | Inter-RAT measurement reporting criteria |  |  |  |  |  |
| - Parameters required for each event |  | 1 entry |  |  |  |  |  |
| - Inter-RAT event identity |  | Set according to specific test. |  |  |  |  |  |
| - Threshold own system |  | -66 dB |  |  |  |  |  |
| - W |  | 0 |  |  |  |  |  |
| - Threshold other system |  | -55 (-80 dBm) |  | When measurement quantity is RSRP, range should be (-115..-19), the actual value = Threshold other system - 25 [dBm] |  |  |  |
| - Hysteresis |  | 0 |  |  |  |  |  |
| - Time to trigger |  | 10 ms |  |  |  |  |  |
| - Reporting cell status |  |  |  |  |  |  |  |
| - CHOICE reported cell |  | Report cells within active set or within virtual active set or of the other RAT |  |  |  |  |  |
| - Maximum number of reported cells |  | 2 |  |  |  |  |  |
| - Idle Interval Information |  | Not present |  |  |  |  |  |
| CELL_DCH measurement occasion info LCR |  |  |  |  |  | LCR_TDD_UTRA |  |
| - cellDCHMeasOccasionSequence |  | 1 entry |  |  |  |  |  |
| - Pattern sequence identifier |  | 0 |  |  |  |  |  |
| - Status Flag |  | activate |  |  |  |  |  |
| - Measurement purpose |  | '00001' |  |  |  |  |  |
| - Measurement occasion pattern sequence parameters |  |  |  |  |  |  |  |
| - k |  | 3 |  |  |  |  |  |
| - Offset |  | 0 |  |  |  |  |  |
| - M_Length |  | 1 |  |  |  |  |  |
| - Timeslot Bitmap |  | Not Present |  |  |  |  |  |
| DPCH Compressed mode status info |  | Not Present |  |  |  | FDD_UTRA AND NOT Compressed_mode |  |
| DPCH Compressed mode status info |  |  |  |  |  | FDD_UTRA AND Compressed_mode |  |
| - TGPS reconfiguration CFN |  | (Current CFN + (250 – TTI/10msec)) mod 256 |  |  |  |  |  |
| - Transmission gap pattern sequence |  | 1 entry |  |  |  |  |  |
| - TGPSI |  | 1 |  |  |  |  |  |
| - TGPS Status Flag |  | activate |  |  |  |  |  |
| - TGCFN |  | (Current CFN + (252 – TTI/10msec)) mod 256 |  |  |  |  |  |
| DPCH Compressed mode status info |  | Not present |  |  |  | TDD_UTRA |  |

| Condition | Explanation |
| --- | --- |
| FDD_UTRA | FDD UTRA cell environment |
| TDD_UTRA | TDD UTRA cell environment |
| LCR_TDD_UTRA | LCR TDD UTRA cell environment |
| Compressed_mode | The UE Support of Compressed mode (TS 36.523-2 table A.4.4-1/28). |
| Band > 64 | If band > 64 is selected |

#### –	MEASUREMENT REPORT

Table 4.7B.1-4: MEASUREMENT REPORT

| Derivation Path: 25.331, clause 10.2.19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Message Type |  |  |  |
| Integrity check info |  |  |  |
| - Message authentication code | This IE is checked to see if it is present. The value is compared against the XMAC-I value computed by SS. The first/ leftmost bit of the bit string contains the most significant bit of the MAC-I. |  |  |
| - RRC Message sequence number | This IE is checked to see if it is present. The value is used by SS to compute the XMAC-I value. |  |  |
| Activation time | (256+CFN-(CFN MOD 8 + 8)) MOD 256 |  |  |
| Measurement identity | 3 |  |  |
| Measured Results | Not present |  |  |
| Measured results on RACH | Not present |  |  |
| Additional measured results | Not present |  |  |
| Event results | Not present |  |  |
| Inter-RAT cell info indication | Not present |  |  |
| E-UTRA Measured Results |  |  |  |
| - E-UTRA measured results list | 1 entry |  |  |
| - E-UTRA Carrier Frequency |  |  |  |
| - Measured E-UTRA cells | 1 entry |  |  |
| - Physical Cell Identity | PhysicalCellIdentity of E-UTRA Cell |  |  |
| - RSRP | Set according to specific test. |  |  |
| - RSRQ | Set according to specific test. |  |  |
| E-UTRA Event Results |  |  |  |
| - Inter-RAT event identity | Set according to specific test. |  |  |
| - E-UTRA events results list | 1 entry |  |  |
| - E-UTRA Carrier Frequency | Downlink EARFCN of E-UTRA Cell |  |  |
| - E-UTRA Carrier Frequency | 65535 |  | Band > 64 |
| - Reported cells | 1 entry |  |  |
| - Physical Cell Identity | PhysicalCellIdentity of E-UTRA Cell |  |  |
| - E-UTRA events results extension list |  |  | Band > 64 |
| - EARFCN extension | Downlink EARFCN of E-UTRA Cell |  |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

#### –	PHYSICAL CHANNEL RECONFIGURATION

Table 4.7B.1-5: PHYSICAL CHANNEL RECONFIGURATION

| Derivation Path: 25.331, clause 10.2.22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Message Type |  |  |  |
| RRC transaction identifier | Arbitrarily selects an unused integer between 0 to 3 |  |  |
| Integrity check info |  |  |  |
| - Message authentication code | SS calculates the value of MAC-I for this message and writes to this IE. The first/ leftmost bit of the bit string contains the most significant bit of the MAC-I. |  |  |
| - RRC message sequence number | SS provides the value of this IE, from its internal counter. |  |  |
| Integrity protection mode info | Not Present |  |  |
| Ciphering mode info | Not Present |  |  |
| Activation time | Not Present |  |  |
| Delay restriction flag | Not Present |  |  |
| New U-RNTI | Not Present |  |  |
| New C-RNTI | Not Present |  |  |
| New H-RNTI | Not Present |  |  |
| New Primary E-RNTI | Not Present |  |  |
| New Secondary E-RNTI | Not Present |  |  |
| RRC State indicator | CELL_DCH |  |  |
| UE Mobility State Indicator | Not Present |  |  |
| UTRAN DRX cycle length coefficient | Not Present |  |  |
| CN information info | Not Present |  |  |
| URA identity | Not Present |  |  |
| RNC support for change of UE capability | Not Present |  |  |
| Reconfiguration in response to requested change of UE capability | Not Present |  |  |
| Downlink counter synchronization info | Not Present |  |  |
| Frequency info | Not Present |  |  |
| DTX-DRX timing information | Not Present |  |  |
| DTX-DRX Information | Not Present |  |  |
| HS-SCCH less Information | Not Present |  |  |
| MIMO parameters | Not Present |  |  |
| HARQ Info | Not Present |  |  |
| Maximum allowed UL TX power | Not Present |  |  |
| CHOICE channel requirement | Not Present |  |  |
| E-DCH Info | Not Present |  |  |
| Downlink HS-PDSCH Information | Not Present |  |  |
| Downlink information common for all radio links |  |  |  |
| - Downlink DPCH info common for all RL | Not Present |  |  |
| - DPCH compressed mode info | Not Present |  | TDD |
| - DPCH compressed mode info |  |  | FDD |
| - TGPSI | 1 |  |  |
| - TGPS Status Flag | Deactivate |  |  |
| - TGCFN | Not Present |  |  |
| - Transmission gap pattern sequence configuration parameters |  |  |  |
| - TGMP | E-UTRA measurement |  |  |
| - TGPRC | Infinity |  |  |
| - TGSN | 8 |  |  |
| - TGL1 | 10 |  |  |
| - TGL2 | Not Present |  |  |
| - TGD | undefined |  |  |
| - TGPL1 | 12 |  |  |
| - TGPL2 | Not Present |  |  |
| - RPP | Mode 0 |  |  |
| - ITP | Mode 0 |  |  |
| - CHOICE UL/DL Mode | UL and DL, UL only or DL only (depending on the UE capability) |  |  |
| - Downlink compressed mode method | HLS(or not sent, depending on the UE capability) |  |  |
| - Uplink compressed mode method | HLS(or not sent, depending on the UE capability) |  |  |
| - Downlink frame type | B |  |  |
| - DeltaSIR1 | 20 (2.0) |  |  |
| - DeltaSIRAfter1 | 10 (1.0) |  |  |
| - DeltaSIR2 | Not Present |  |  |
| - DeltaSIRAfter2 | Not Present |  |  |
| - N identify abort | Not Present |  |  |
| - T Reconfirm abort | Not Present |  |  |
| - TX Diversity mode | Not Present |  |  |
| - SSDT information | Not Present |  |  |
| - Default DPCH Offset Value | Not Present |  |  |
| Downlink information for each radio link | Not Present |  |  |
| MBMS PL Service Restriction Information | Not Present |  |  |

#### –	PHYSICAL CHANNEL RECONFIGURATION COMPLETE

Table 4.7B.1-6: PHYSICAL CHANNEL RECONFIGURATION COMPLETE

| Derivation Path: 25.331, clause 10.2.23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Message Type |  |  |  |
| Integrity check info |  |  |  |
| - Message authentication code | This IE is checked to see if it is present. The value is compared against the XMAC-I value computed by SS. The first/ leftmost bit of the bit string contains the most significant bit of the MAC-I. |  |  |
| - RRC Message sequence number | This IE is checked to see if it is present. The value is used by SS to compute the XMAC-I value. |  |  |
| Uplink integrity protection activation info | Not checked |  |  |
| COUNT-C activation time | Not checked |  |  |
| Uplink counter synchronization info | Not checked |  |  |

#### –	RRC CONNECTION REQUEST

Table 4.7B.1-7: RRC CONNECTION REQUEST

| Derivation Path: 25.331, clause 10.2.39 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Message Type |  |  |  |
| Predefined configuration status information | To be checked against requirement if specified |  |  |
| Initial UE identity |  |  |  |
| - CHOICE UE id type |  |  |  |
| - TMSI and LAI (GSM-MAP) | Set to the UE's TMSI and LAI. |  |  |
| Establishment cause | To be checked against requirement if specified |  |  |
| Protocol error indicator | FALSE |  |  |
| UE Specific Behaviour Information 1 idle | This IE will not be checked by default behaviour, but in specific test case. |  |  |
| Domain indicator | Not checked |  |  |
| Call type | Not checked |  |  |
| UE capability indication | Not checked |  |  |
| MBMS Selected Services | Not checked |  |  |
| Support for F-DPCH | Not checked |  |  |
| UE Mobility State Indicator | Not Present |  |  |
| Support for Enhanced F-DPCH | Not checked |  |  |
| HS-PDSCH in CELL_FACH | Not checked |  |  |
| MAC-ehs support | Not checked |  |  |
| DPCCH Discontinuous Transmission support | Not checked |  |  |
| Support of common E-DCH | Not checked | REL-8 |  |
| Multi cell support | Not checked | REL-8 |  |
| Pre-redirection info | Not checked | REL-8 |  |
| Support of MAC-i/is | Not checked | REL-8 |  |
| Support of SPS operation | Not checked | REL-8 |  |
| Support for CS Voice over HSPA | Not checked | REL-8 |  |
| Dual cell MIMO support | Not checked | REL-9 |  |
| System Information Container Stored Indicator | Not checked | REL-9 |  |
| Measured results on RACH | Not checked |  |  |
| Access stratum release indicator | Not checked |  |  |

#### –	SECURITY MODE COMMAND

Table 4.7B.1-n: SECURITY MODE COMMAND

| Derivation Path: 25.331, clause 10.2.43 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FFS |  |  |  |

#### –	SECURITY MODE COMPLETE

Table 4.7B.1-n: SECURITY MODE COMPLETE

| Derivation Path: 25.331, clause 10.2.44 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FFS |  |  |  |

#### –	UTRAN MOBILITY INFORMATION

Table 4.7B.1-n: UTRAN MOBILITY INFORMATION

| Derivation Path: 34.108 clause 9.1.1 (UTRAN MOBILITY INFORMATION message) |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UTRAN MOBILITY INFORMATION |  |  |  |  |
| CN information info |  |  |  |  |
| - PLMN identity |  | Not present |  |  |
| - CN common GSM-MAP NAS system information |  |  |  |  |
| - GSM-MAP NAS system information |  | 00 01H |  |  |
| - CN domain related information |  |  |  |  |
| - CN domain identity |  | PS |  |  |
| - CN domain specific NAS system information |  |  |  |  |
| - GSM-MAP NAS system information |  | 01 00H |  |  |
| - CN domain specific DRX cycle length coefficient |  | 7 |  |  |
| - CN domain identity |  | CS |  |  |
| - CN domain specific NAS system information |  |  |  |  |
| - GSM-MAP NAS system information |  | 1E 01H |  |  |
| - CN domain specific DRX cycle length coefficient |  | 7 |  |  |

#### –	UTRAN MOBILITY INFORMATION CONFIRM

Table 4.7B.1-n: UTRAN MOBILITY INFORMATION CONFIRM

Derivation Path: 34.108 clause 9.1.1 (UTRAN MOBILITY INFORMATION CONFIRM message)

### 4.7B.2	UTRA NAS messages

This section contains default message contents for UTRA NAS messages.

In 3GPP TS 36.523-1 [18]:

-	UTRA NAS messages are defined as exceptions, IE by IE, to the messages defined in this section;

-	UTRA NAS messages not defined in this section are specified completely.

The following messages are not specified in this section:

-	AUTHENTICATION REQUEST (CS)

-	AUTHENTICATION RESPONSE (CS)

-	DETACH REQUEST (CS & PS)

-	DETACH ACCEPT (CS & PS)

-	IDENTITY REQUEST (CS & PS)

-	IDENTITY RESPONSE (CS & PS)

-	P-TMSI REALLOCATION COMMAND (PS)

-	P-TMSI REALLOCATION COMPLETE (PS)

-	SERVICE REQUEST (PS)

Table 4.7B.2-1: ROUTING AREA UPDATE REQUEST

| Derivation path: 24.008 table 9.4.14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Update type | 000 'RA updating' or 001 'combined RA/LA updating' 010 ‘combined RA/LA updating with IMSI attach’ | In NMO I, depending on capability and domain preference for voice and SMS, the UE may initiate either PS only or CS/PS registration procedures. |  |
| GPRS ciphering key sequence number | Any allowed value |  |  |
| Old routing area identification | Any allowed value |  |  |
| MS Radio Access capability | Any allowed value | The checking of this IE is expected to be covered in UMTS test cases. |  |
| Old P-TMSI signature | Not present or any allowed value | For test cases where no "UE" statement indicates that a P-TMSI signature was previously registered, whether or not the UE includes this IE depends on previous USIM contents. |  |
| Requested READY timer value | Not present or any allowed value | The checking of this IE is expected to be covered in UMTS test cases. |  |
| DRX parameter | Not present or any allowed value | The checking of this IE is expected to be covered in UMTS test cases. |  |
| TMSI status | 0 'no valid TMSI available' or not present |  |  |
| P-TMSI | Not present or any allowed value |  |  |
| MS network capability | Any allowed value | The checking of this IE is expected to be covered in UMTS test cases. |  |
| PDP context status | Any allowed value | The checking of this IE is expected to be covered in UMTS test cases. |  |
| PS LCS Capability | Not present or any allowed value | The checking of this IE is expected to be covered in UMTS test cases. |  |
| MBMS context status | Not present |  |  |
| UE network capability | Any allowed value (must be present) | No detailed checking by default but should be aligned with PICS |  |
| Additional mobile identity | Not present or any allowed value |  |  |
| Additional old routing area identification | Not present or any allowed value |  |  |
| Mobile station classmark 2 | Not present or any allowed value | This IE shall be included if the MS supports SRVCC to GERAN or UTRAN. |  |
| Mobile station classmark 3 | Not present or any allowed value | This IE shall be included if the MS supports SRVCC to GERAN. |  |
| Supported Codecs | Not present or any allowed value |  |  |
| Voice domain preference and UE's usage setting | Not present or any allowed value | Rel-9 |  |
| P-TMSI type | Any allowed value | Rel-10 |  |

Table 4.7B.2-2: ROUTING AREA UPDATE ACCEPT

| Derivation path: 24.008 table 9.4.15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Force to standby | 0 'Force to standby not indicated' |  |  |
| Update result | 000 'RA updated' |  | IF the UE indicated 000 'RA updating' in the previous RAU request message |
|  | 001 'combined RA/LA updated' |  | IF the UE indicated 001 'combined RA/LA updating' in the previous RAU request message |
|  | 1 ‘No follow-on proceed’ |  |  |
| Periodic RA update timer | 111 (deactivated) |  |  |
| Routing area identification | 1 | Value in table 4.4.4-2 for simulated UTRA cells. |  |
| P-TMSI signature | Arbitrary value |  |  |
| Allocated P-TMSI | Arbitrary value | 2 most significant bits = 11 to indicate SGSN |  |
| MS identity | Not present | Is never present in case of RA only procedure, and RA/LA procedures do not necessarily change TMSI |  |
| List of Receive N PDU Numbers | Not present | Realistic network value |  |
| Negotiated READY timer value | Not present |  |  |
| GMM cause | Not present |  |  |
| T3302 value | Not present |  |  |
| Cell Notification | Not present | This IE is for A/Gb mode only. |  |
| Equivalent PLMNs | Not present |  |  |
| PDP context status | Same value as the one received in the RAU request message |  | UTRAN |
| PDP context status | 0 | NSAPI(0) - NSAPI(15) is set to 0, which means that the SM state of all PDP contexts is PDP-INACTIVE | GERAN |
| Network feature support | Not present |  |  |
| Emergency Number List | Not present |  |  |
| MBMS context status | Not present |  |  |
| Requested MS Information | Not present |  |  |
| T3319 value | Not present |  |  |
| T3323 value | Not present |  |  |

| Condition | Explanation |
| --- | --- |
| UTRAN | UTRAN cell environment |
| GERAN | GERAN cell environment |

Table 4.7B.2-3: ROUTING AREA UPDATE COMPLETE

| Derivation path: 24.008 table 9.4.16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| List of Receive N PDU Numbers | Not present |  |  |
| Inter RAT handover information | Not present |  |  |
| E-UTRAN inter RAT handover information | Not present |  |  |

Table 4.7B.2-4: LOCATION UPDATING REQUEST

| Derivation path: 24.008 table 9.2.17 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Location updating type | Any allowed value |  |  |
| Ciphering key sequence number | Any allowed value |  |  |
| Location area identification | Any allowed value |  |  |
| Mobile station classmark | Any allowed value |  |  |
| Mobile identity | Any allowed value |  |  |
| Mobile station classmark for UMTS | Any allowed value |  |  |
| Additional update parameters | Not present or any allowed value |  |  |

Table 4.7B.2-5: LOCATION UPDATING ACCEPT

| Derivation path: 24.008 table 9.2.15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Location area identification | 1 |  |  |
| Mobile identity |  |  |  |
| TMSI | Arbitrary value | 2 most significant bits = 00 to indicate VLR |  |
| Follow on proceed | Present | This IE is unncessary in non-CS fallback test cases. |  |
| CTS permission | Absent |  |  |
| Equivalent PLMNs | Absent |  |  |
| Emergency Number List | Absent |  |  |

Table 4.7B.2-6: ATTACH REQUEST

| Derivation path: 24.008 table 9.4.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| MS network capability | Any allowed value |  |  |
| Attach type | 001 'GPRS attach' or 011 'Combiner GPRS/IMS attach' |  |  |
| GPRS ciphering key sequence number | Any allowed value |  |  |
| DRX parameter | Any allowed value |  |  |
| P-TMSI or IMSI | Any allowed value |  |  |
| Old routing area identification | Any allowed value |  |  |
| MS Radio Access capability | Any allowed value |  |  |
| Old P-TMSI signature | Not present or any allowed value |  |  |
| Requested READY timer value | Not present or any allowed value |  |  |
| TMSI status | 0 'no valid TMSI available' or not present |  |  |
| PS LCS Capability | Not present or any allowed value |  |  |
| Mobile station classmark 2 | Not present or any allowed value |  |  |
| Mobile station classmark 3 | Not present or any allowed value |  |  |
| Supported Codecs | Not present or any allowed value |  |  |
| UE network capability | Any allowed value (must be present) |  |  |
| Additional mobile identity | Not present or any allowed value |  |  |
| Additional old routing area identification | Not present or any allowed value |  |  |
| Voice domain preference and UE's usage setting | Not present or any allowed value | Rel-9 |  |
| P-TMSI type | Not present or any allowed value | Rel-10 |  |

Table 4.7B.2-7: ATTACH ACCEPT

| Derivation path: 24.008 table 9.4.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Attach result | 001 'GPRS only attached' |  | IF the UE indicated 001 'GPRS attach' in the previous attach request message |
|  | 011 'combined GPRS/IMSI attached' |  | IF the UE indicated 011 'Combined GPRS/IMSI attach' in the previous attach request message |
| Force to standby | 000 'Force to standby not indicated' |  |  |
| Periodic RA update timer | 111 (deactivated) |  |  |
| Radio priority for SMS | 100 'priority level 4 (lowest)' |  |  |
| Radio priority for TOM8 | 100 'priority level 4 (lowest)' |  |  |
| Routing area identification | 1 |  |  |
| P-TMSI signature | Arbitrary value |  |  |
| Negotiated READY timer value | Not present |  |  |
| Allocated P-TMSI | Arbitrary value | 2 most significant bits = 11 to indicate SGSN |  |
| MS identity | Not present |  | IF the UE indicated 001 'GPRS attach' in the previous attach request message |
|  | Arbitrary value | 2 most significant bits = 00 to indicate VLR | IF the UE indicated 011 'Combined GPRS/IMSI attach' in the previous attach request message |
| GMM cause | Not present |  |  |
| T3302 value | Not present |  |  |
| Cell Notification | Not present |  |  |
| Equivalent PLMNs | Not present |  |  |
| Network feature support | Not present |  |  |
| Emergency Number List | Not present |  |  |
| Requested MS Information | Not present |  |  |
| T3319 value | Not present |  |  |
| T3323 value | Not present |  |  |

Table 4.7B.2-8: ATTACH COMPLETE

| Derivation path: 24.008 table 9.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Inter RAT handover information | Not present |  |  |
| E-UTRAN inter RAT handover information | Not present |  |  |

Table 4.7B.2-9: AUTHENTICATION AND CIPHERING REQUEST

| Derivation path: 24.008 table 9.4.9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Ciphering algorithm | Any allowed value |  |  |
| IMEISV request | 000 ‘IMEISV not requested’ |  |  |
| Force to standby | 000 'Force to standby not indicated' |  |  |
| A&C reference number | Any allowed value |  |  |

Table 4.7B.2-10: AUTHENTICATION AND CIPHERING RESPONSE

| Derivation path: 24.008 table 9.4.10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| A&C reference number | Arbitrary value |  |  |
| Spare half octet | 0000 |  |  |

Table 4.7B.2-11: TMSI REALLOCATION COMMAND

| Derivation path: 24.008 table 9.2.20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Location area identification | Any allowed value |  |  |
| Mobile identity | Any allowed value |  |  |

Table 4.7B.2-12: TMSI REALLOCATION COMPLETE

Derivation path: 24.008 table 9.2.21

Table 4.7B.2-13: CM SERVICE REQUEST

| Derivation path: 24.008 table 9.2.11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| CM service type | Any allowed value |  |  |
| Ciphering key sequence number | Any allowed value |  |  |
| Mobile station classmark | Any allowed value |  |  |
| Mobile identity | Any allowed value |  |  |
| Priority | Not present or any allowed value |  |  |
| Additional update parameters | Not present or any allowed value |  |  |

## 4.7C	Default DS-MIPv6 message and information element contents

### 4.7C.1	IKEv2 messages

#### -	IKEv2 IKE_SA_INIT Request

This message is sent by the UE to the SS.

Table 4.7C.1-1: Message IKE_SA_INIT Request

| Field | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| IKE Header |  |  |  |
| Initiator’s IKE_SA SPI | Set by the UE |  |  |
| Responder’s IKE_SA SPI | 0 | First message jn IKE_SA_INIT exchange |  |
| Next Payload | ‘00100001’B | SA |  |
| Exchange Type | ‘00100010’B | IKE_SA_INIT |  |
| Security Association Payload |  |  |  |
| Next Payload | ’00100010’B | KE |  |
| More proposal | ‘00000010’B |  |  |
| Proposal # | ‘00000001’B | First cryptographic suite (section 6.5 of TS 33.234) |  |
| Protocol ID | ‘00000001’B | IKE |  |
| SPI size | ‘00000000’B |  |  |
| Number of transforms | ‘00000010’B |  |  |
| More transform | ‘00000011’B | This is the transform for confidentiality |  |
| Transform type | ‘00000001’B | Encryption |  |
| Transform ID | ‘00000011’B | 3DES in CBC mode (ENCR_3DES) |  |
| More transform | ‘00000011’B | This is the transform for prf |  |
| Transform type | ‘00000010’B | PRF |  |
| Transform ID | ‘00000010’B | PRF_HMAC_SHA1 (HMAC-SHA1) |  |
| More transform | ‘00000011’B | This is the transform for integrity |  |
| Transform type | ‘00000011’B | Integrity |  |
| Transform ID | ‘00000010’B | HMAC-SHA1-96 (AUTH_HMAC_SHA1_96) |  |
| Last transform | ‘00000000’B | This is the transform for DH |  |
| Transform type | ‘00000100’B | DH |  |
| Transform ID | ‘00000010’B | Diffie-Hellman group 2 (1024-bit MODP) |  |
| Last proposal | ‘00000000’B |  |  |
| Proposal # | ‘00000010’B | Second cryptographic suite (section 6.5 of TS 33.234) |  |
| Protocol ID | ‘00000001’B | IKE |  |
| SPI size | ‘00000000’B |  |  |
| Number of transforms | ‘00000010’B |  |  |
| More transform | ‘00000011’B | This is the transform for confidentiality |  |
| Transform type | ‘00000001’B | Encryption |  |
| Transform ID | ‘00001011’B | AES with 128-bit keys in CBC mode (ENCR_AES_CBC) |  |
| More transform | ‘00000011’B | This is the transform for prf |  |
| Transform type | ‘00000010’B | PRF |  |
| Transform ID | ‘00000100’B | PRF_AES128_XCBC_AES-XCBC-PRF-128 |  |
| More transform | ‘00000011’B | This is the transform for integrity |  |
| Transform type | ‘00000011’B | Integrity |  |
| Transform ID | ‘00000101’B | AES-XCBC-MAC-96 (AUTH_AES-XCBC -96) |  |
| Last transform | ‘00000000’B | This is the transform for DH |  |
| Transform type | ‘00000100’B | DH |  |
| Transform ID | ‘00000010’B | Diffie-Hellman group 2 (1024-bit MODP) |  |
| Key Exchange Payload |  |  |  |
| Next Payload | ‘00101000’B | Nonce |  |
| DH Group # | ‘0000000000000010’B | DH group 2 |  |
| Key Exchange data | Set by the UE |  |  |
| Nonce Payload |  |  |  |
| Next Payload | ‘00101001’B | Notify | UE IKE INIT HA |
| Nonce data | Random number set by the UE |  |  |
| REDIRECT_SUPPORTED Notify Payload |  |  | UE IKE INIT HA |
| Next Payload | ‘00000000’B | No Next Payload | UE IKE INIT HA |
| Protocol ID | ‘00000000’B | Notification is not specific to a particular security association | UE IKE INIT HA |
| SPI size | ‘00000000’B | SPI field not present | UE IKE INIT HA |
| Notify Message Type | '0100000000010110'B | REDIRECT_SUPPORTED | UE IKE INIT HA |
| REDIRECT_FROM Notify Payload |  |  | UE IKE INIT HA Redirected |
| Next Payload | ‘00000000’B | No next payload | UE IKE INIT HA Redirected |
| Protocol ID | ‘00000000’B | Notification is not specific to a particular security association | UE IKE INIT HA Redirected |
| SPI size | ‘00000000’B | SPI field not present | UE IKE INIT HA Redirected |
| Notify Message Type | '0100000000010110'B | REDIRECT_FROM | UE IKE INIT HA Redirected |
| GW Ident Type | Any allowed value (IPv6 or IPv4 or HA FQDN) | Set depending on how the UE has discovered the HA in the preamble | UE IKE INIT HA Redirected |
| New Responder GW Identity | Depends on GW Ident type |  | UE IKE INIT HA Redirected |

| Condition | Explanation |
| --- | --- |
| UE IKE INIT HA | Part of IKE_INIT_SA request message sent to HA only |
| UE IKE INIT HA Redirected | Part of IKE_INIT_SA request message sent to HA only after a REDIRECT payload was received in a previous IKEV2 messages exchange |

#### -	IKE_SA_INIT Response

This message is sent by the SS to the UE.

Table 4.7C.1-2: Message IKE_SA_INIT Response

| Field | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| IKE Header |  |  |  |
| Initiator’s IKE_SA SPI | Same as that set by the UE in IKE_SA_INIT request |  |  |
| Responder’s IKE_SA SPI | Set by the SS |  |  |
| Next Payload | ‘00100001’B | SA |  |
| Exchange Type | ‘00100010’B | IKE_SA_INIT |  |
| Security Association Payload |  |  |  |
| Next Payload | ’00100010’B | KE |  |
| Proposal | One of the 2 proposals included in IKE_SA_INIT at Step 1 |  |  |
| Key Exchange Payload |  |  |  |
| Next payload | ‘00 101000’B | Nonce |  |
| DH Group # | ‘0000000000000010’B | DH group 2 |  |
| Key Exchange data | Set by the SS |  |  |
| Nonce Payload |  |  |  |
| Next t payload | ‘00000000’B | No Next Payload |  |
| Nonce data | Set by the SS |  |  |

#### -	IKE_AUTH_Request

This message is sent by the UE to the SS.

Table 4.7C.1-3: Message IKE_AUTH Request

| Field | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| IKE Header |  |  |  |
| Initiator’s IKE_SA SPI | Same as that set by the UE in IKE_INIT_SA Request |  |  |
| Responder’s IKE_SA SPI | Same as that set by the SS in IKE_INIT_SA Response |  |  |
| Next Payload | ‘00101110’B | E |  |
| Exchange Type | ‘00100011’B | IKE_AUTH |  |
| Encrypted Payload |  |  |  |
| Next Payload | ‘00100011’B | IDi | UE IKE AUTH1 |
| Initialization Vector | Random value set by the UE |  | UE IKE AUTH1 |
| Encrypted IKE Payloads |  |  | UE IKE AUTH1 |
| Identification – Initiator Payload |  |  | UE IKE AUTH1 |
| Next Payload | ‘00101111’B | CP | UE IKE AUTH1 |
| ID Type | 00000010B |  | UE IKE AUTH1 |
| ID | Set to MN-NAI |  | UE IKE AUTH1 |
| Configuration Payload |  |  | UE IKE AUTH1 |
| Next Payload | ‘00100001’B | SA | UE IKE AUTH1 |
| CFG Type | ‘00000001’B | Request | UE IKE AUTH1 |
| Configuration Attribute | ‘00010000’B | MIP6_HOME_PREFIX attribute | UE IKE AUTH1 HA |
| Length | ‘0000000000000000’B |  | UE IKE AUTH1 HA |
| Security Association Payload |  |  | UE IKE AUTH1 |
| Next Payload | ‘00101100’B | TSi | UE IKE AUTH1 |
| Proposals | Any set of allowed values |  | UE IKE AUTH1 |
| Traffic Selector – Initiator Payload |  |  | UE IKE AUTH1 |
| Next Payload | ‘00101100’B | TSr | UE IKE AUTH1 |
| Traffic selector data | Any allowed set of values |  | UE IKE AUTH1 |
| Traffic Selector – Responder Payload |  |  | UE IKE AUTH1 |
| Next Payload | ‘00100100’B | IDr | UE IKE AUTH1 |
| Traffic selector data | Any allowed set of values |  | UE IKE AUTH1 |
| Identification – Responder Payload |  |  | UE IKE AUTH1 |
| Next Payload | ‘00000000’B | No Next Payload | UE IKE AUTH1 |
| ID Type | ‘00000010’B |  | UE IKE AUTH1 |
| ID | APN |  | UE IKE AUTH1 |
| Next Payload | ‘00110000’B | EAP | UE IKE AUTH2 |
| Initialization Vector | Random value set by the UE |  | UE IKE AUTH2 |
| Encrypted IKE Payloads |  |  | UE IKE AUTH2 |
| Extensible Authentication Payload |  |  | UE IKE AUTH2 |
| Next Payload | ‘00000000’B | No Next Payload | UE IKE AUTH2 |
| Code | ‘00000010’B | Response | UE IKE AUTH2 |
| Type | ‘00010111’B | AKA | UE IKE AUTH2 |
| Subtype |  | AKA-Challenge | UE IKE AUTH2 |
| Attribute type | ’00000011’B | AT_RES | UE IKE AUTH2 |
| AT_RES | See TS 24.301 [28] subclause 9.9.3.4 |  | UE IKE AUTH2 |
| Next Payload | ‘00100111’B | AUTH | UE IKE AUTH3 |
| Initialization Vector | Random value set by the UE |  | UE IKE AUTH3 |
| Encrypted IKE Payloads |  |  | UE IKE AUTH3 |
| Authentication Payload |  |  | UE IKE AUTH3 |
| Next Payload | ‘00000000’B | No Next Payload | UE IKE AUTH3 |
| Auth Method | ’00000010’B | Shared Key Integrity code | UE IKE AUTH3 |
| Auth Data | derived from the MSK obtained from AKA exchange | RFC 4306 defines the function to derive this key (section 2.15) | UE IKE AUTH3 |
| Padding | Set by the UE | Fields from Encrypted payload |  |
| Pad Length | Set by the UE | Fields from Encrypted payload |  |
| Integrity checksum data | Set by the UE | Fields from Encrypted payload |  |

| Condition | Explanation |
| --- | --- |
| UE IKE AUTH1 | Part of IKE_AUTH request message sent to HA or ePDG after the reception of IKE_INIT_SA response |
| UE IKE AUTH1 HA | Part of IKE_AUTH request message sent to HA after the reception of IKE_INIT_SA response |
| UE IKE AUTH2 | Part of IKE_AUTH request message sent to HA or ePDG after the reception of IKE_SA response message containing the certificate payload CERT |
| UE IKE AUTH3 | Part of IKE_AUTH request message sent to HA or ePDG after the reception of IKE_SA response containing EAP and encryption payloads only |

#### -	IKE_AUTH Response

This message is sent by the SS to the UE.

Table 4.7C.1-4: Message IKE_AUTH Response

| Field | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| IKE Header |  |  |  |
| Initiator’s IKE_SA SPI | Same as that set by the UE in IKE_SA_INIT request |  |  |
| Responder’s IKE_SA SPI | Same as that set by the SS in IKE_SA_INIT response |  |  |
| Next Payload | ‘00101110’B | E |  |
| Exchange Type | ‘00100011’B | IKE_AUTH |  |
| Encrypted Payload |  |  |  |
| Next Payload | ‘00100100’B | IDr | SS IKE AUTH1 |
| Initialization Vector | Set by the SS |  | SS IKE AUTH1 |
| Encrypted IKE Payloads |  |  | SS IKE AUTH1 |
| Identification – Responder Payload |  |  | SS IKE AUTH1 |
| Next Payload | ‘00100101’B | CERT | SS IKE AUTH1 |
| ID Type | ‘00000010’B |  | SS IKE AUTH1 |
| ID | APN |  | SS IKE AUTH1 |
| Certificate Payload |  |  | SS IKE AUTH1 |
| Next Payload | ‘00110000’B | EAP | SS IKE AUTH1 |
| Cert encoding | ’00000100’B | X.509 certificate - signature | SS IKE AUTH1 |
| Certificate data | Set by the SS | DER encoded X.509 certificate | SS IKE AUTH1 |
| Extensible Authentication Payload |  |  | SS IKE AUTH1 |
| Next Payload | ‘00000000’B | No Next Payload | SS IKE AUTH1 |
| Code | ‘00000001’B | Request | SS IKE AUTH1 |
| Type | ‘00010111’B | AKA | SS IKE AUTH1 |
| Subtype |  | AKA-Challenge | SS IKE AUTH1 |
| Attribute type | ’00000001’B | AT_RAND | SS IKE AUTH1 |
| AT_RAND | An arbitrarily selected 128 bits value |  | SS IKE AUTH1 |
| Attribute Type | ‘00000010’B | AT_AUTN | SS IKE AUTH1 |
| AT_AUTN | See TS 24.301 [28] subclause 9.9.3.2 |  | SS IKE AUTH1 |
| Next Payload | ‘00110000’B | EAP | SS IKE AUTH2 |
| Initialization Vector | Set by the SS |  | SS IKE AUTH2 |
| Encrypted IKE Payloads |  |  | SS IKE AUTH2 |
| Extensible Authentication Payload |  |  | SS IKE AUTH2 |
| Next Payload | ‘00000000’B | No Next Payload | SS IKE AUTH2 |
| Code | ‘00000011’B | Success | SS IKE AUTH2 |
| Next Payload | ‘00101111’B | CP | SS IKE AUTH3 |
| Auth Method | ’00000010’B | Shared Key Integrity code | SS IKE AUTH3 |
| Auth Data | derived from the MSK obtained from AKA exchange | RFC 4306 defines the function to derive this key (section 2.15) | SS IKE AUTH3 |
| Configuration Payload |  |  | SS IKE AUTH3 |
| Next Payload | ‘00100001’B | SA | SS IKE AUTH3 |
| CFG Type | ‘00000010’B | Reply | SS IKE AUTH3 |
| Configuration Attribute | ‘00010000’B | MIP6_HOME_PREFIX attribute | SS IKE AUTH3 HA |
| Length | ‘0000000000010101’B |  | SS IKE AUTH3 HA |
| Prefix lifetime | Any allowed value |  | SS IKE AUTH3 HA |
| Home Prefix | IPv6 prefix – 16 bytes |  | SS IKE AUTH3 HA |
| Prefix length | ‘10000000’B | Prefix length must be 64 | SS IKE AUTH3 HA |
| Notify Payload |  |  | SS IKE AUTH3 HA redirect |
| Next Payload | ‘00100001’B | SA | SS IKE AUTH3 HA redirect |
| Protocol ID | ‘00000000’B | Notification is not specific to a particular security association | SS IKE AUTH3 HA redirect |
| SPI Size | ‘00000000’B | SPI field not present | SS IKE AUTH3 HA redirect |
| Notify Message Type Length | ‘0100000000010111’B | REDIRECT | SS IKE AUTH3 HA redirect |
| GW Ident Type | ‘00000101’B |  | SS IKE AUTH3 HA redirect |
| New Responder GW Identity | IPv6 address of the HA to relocate |  | SS IKE AUTH3 HA redirect |
| GW Ident Type | ‘00000001’B |  | SS IKE AUTH3 HA redirect |
| New Responder GW Identity | IPv4 address of the HA to relocate | Optional | SS IKE AUTH3 HA redirect |
| Security Association Payload |  |  | SS IKE AUTH3 |
| Next Payload | ‘00101101’ | TSi | SS IKE AUTH3 |
| Proposal | One of the 2 proposals included in IKE_AUTH Request at Step 3 |  | SS IKE AUTH3 |
| Traffic Selector – Initiator Payload |  |  | SS IKE AUTH3 |
| Next Payload | ‘00101100’B | TSr | SS IKE AUTH3 |
| Traffic Selector data | Any allowed set of values |  | SS IKE AUTH3 |
| Traffic Selector – Responder Payload |  |  | SS IKE AUTH3 |
| Next Payload | ‘00000000’B | No Next Payload | SS IKE AUTH3 |
| Padding | Set by the SS | Fields from Encryption payload |  |
| Pad Length | Set by the SS | Fields from Encryption payload |  |
| Integrity checksum data | Set by the SS | Fields from Encryption payload |  |

| Condition | Explanation |
| --- | --- |
| SS IKE AUTH1 | Part of IKE_AUTH request message sent to the UE as part of the first IKE_AUTH Response message |
| SS IKE AUTH2 | Part of IKE_AUTH request message sent to the UE as part of the second IKE_AUTH Response message |
| SS IKE AUTH3 | Part of IKE_AUTH request message sent to the UE as part of the third IKE_AUTH Response message |
| SS IKE AUTH3 HA | Part of IKE_AUTH request message sent to the UE as part of the third IKE_AUTH Response message when the UE interacts with HA |
| SS IKE AUTH3 HA_redirect | Part of IKE_AUTH request message sent to the UE as part of the third IKE_AUTH Response message when the UE interacts with HA and the UE gets redirected |

### 4.7C.2	Messages used to perform DS-MIPv6 registration and deregistration

#### -	Router Advertisement

This message is sent by the SS to the UE.

Table 4.7C.2-1: Message Router Advertisement

| Field | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| Type | ‘10000110’B |  |  |
| Code | ‘00000000’B |  |  |
| Checksum | Set by SS | ICMP checksum |  |
| Cur Hop Limit | ‘00000000’B | Unspecified |  |
| M | Set by the SS |  |  |
| O | Set by the SS | Depends on access network specific settings |  |
| H | ‘0’B |  |  |
| Prf | Set by the SS | Depends on access network specific settings |  |
| Prf | Set by the SS | Depends on access network specific settings |  |
| P | Set by the SS | Depends on access network specific settings |  |
| Reserved | ‘00000’B |  |  |
| Router Lifetime | Set by the SS |  |  |
| Reachable Time | Set by the SS |  |  |
| Retrans Timer | Set by the SS |  |  |
| type | ‘00000011’B | Prefix Information Option |  |
| Length | ‘00000100’B |  |  |
| Prefix length | Set by the SS |  |  |
| L | ‘1’B |  |  |
| A | Set by the SS |  |  |
| Valid Lifetime | Set by the SS |  |  |
| Preferred Lifetime | Set by the SS |  |  |
| Prefix | Set as per specific message content |  |  |

#### -	Binding Update

This message is sent by the UE to the SS.

Table 4.7C.2-2: Message Binding Update

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| IPv4 Source Address | UE IPv4 CoA (IPv4 address acquired by UE during network attachment) |  | UE IPv4 |
| IPv4 Destination Address | IPv4 of Home Agent discovered during preamble |  | UE IPv4 |
| UDP header |  |  | UE IPv4 |
| Source Port | Set by UE |  | UE IPv4 |
| Destination port | ‘0001000001011111’B |  | UE IPv4 |
| IPv6 Source Address | UE IPv6 CoA (IPv6 address acquired by the UE during network attachment) |  | IPv6 visited |
|  | IPv6 Home Address configured by the UE from Home Network Prefix assigned to UE during preamble |  | UE IPv6-home, UE IPv4 |
| IPv6 Destination Address | IPv6 of Home Agent discovered during preamble |  |  |
| Destination Header | IPv6 Home Address configured by the UE from Home Network Prefix assigned to the UE during preamble |  | IPv6 visited |
| Payload Proto | ‘00111011’B |  |  |
| MH Type | ‘00000101’B | Binding Update message |  |
| Sequence Number | Any allowed value |  |  |
| Lifetime | Any allowed non-zero value |  | IPv6 visited |
|  | ‘0000000000000000’B |  | UE IPv6-home |
| A | ‘1’B |  |  |
| H | ‘1’B |  |  |
| L | Not checked |  |  |
| K | ‘1’B |  |  |
| M | ’0’B |  |  |
| R | ’1’B |  |  |
| P | ‘0’B |  |  |
| F | ’0’B |  |  |
| IPv4 Home Address option | Set to the value "0.0.0.0" to request allocation for the UE. The "P" flag is set to '0'B. The Prefix Length is set to the requested prefix length of '32'. | Optional |  |
| Alternate Care-of Address option | Same IPv6 address as that inserted in the IP Source Address field |  | UE IPv6-visited |

| Condition | Explanation |
| --- | --- |
| UE IPv4 | UE is in an IPv4 visited network (see RFC 5555) |
| UE IPv6-home | UE is in an IPv6 home network (see RFC 5555) |
| UE IPv6-visited | UE is in an IPv6 visited network (see RFC 5555) |

#### -	Binding Acknowledgement

This message is sent by the SS to the UE.

Table 4.7C.2-3: Message Binding Acknowledgement

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| IPv4 Source Address | IPv4 Home Agent address |  | UE IPv4 |
| IPv4 Destination Address | Same value as UE IPv4 CoA in IP Source Address from Binding Update |  | UE IPv4 |
| UDP header |  |  | UE IPv4 |
| Source Port | ‘0001000001011111’B |  | UE IPv4 |
| Destination port | Same as Source port in Binding Update |  | UE IPv4 |
| IPv6 Source Address | IPv6 Home Agent address |  |  |
| IPv6 Destination Address | Same value as UE IPv6 CoA in IP Source Address from Binding Update |  | UE IPv6-visited |
|  | IPv6 Home Address |  | UE IPv6-home, UE IPv4 |
| Routing Header | Same value as UE IPv6 Home Address in Destination Header from Binding Update |  | UE IPv6-visited |
| Payload Proto | ‘00111011’B |  |  |
| MH Type | ‘00000110’B | Binding Acknowledgement message |  |
| Status | ‘00000000’B | Binding Update accepted |  |
| K | Set by the SS |  |  |
| R | ‘1’B |  |  |
| P | ‘0’B |  |  |
| Sequence Number | Same value as that sent by the UE in the Binding Update |  |  |
| Lifetime | ‘0000000010010110’B | 10 min | UE IPv6-visited, UE IPv4 |
|  | ‘0000000000000000’B |  | UE IPv6-home |
| IPv4 Address Acknowledgement option | IPv4 Home Address allocated to the UE | Present if IPv4 Home Address option was included by the UE in Binding Update |  |
| Binding Refresh Advice option | ‘0000000010010110’B | 10 min |  |

| Condition | Explanation |
| --- | --- |
| UE IPv4 | UE is in an IPv4 visited network (see RFC 5555) |
| UE IPv6-home | UE is in an IPv6 home network (see RFC 5555) |
| UE IPv6-visited | UE is in an IPv6 visited network (see RFC 5555) |

#### -	Binding Revocation Indication

This message is sent by the SS to the UE.

Table 4.7C.2-4: Binding Revocation Indication

| Field | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| IPv4 Header |  |  | UE IPv4 |
| IPv4 Source Address | UE IPv4 Home Agent address |  | UE IPv4 |
| UDP header |  |  | UE IPv4 |
| Source Port | Set by SS |  | UE IPv4 |
| Destination port | ‘0001000001011111’B |  | UE IPv4 |
| IPv6 Header |  |  |  |
| IPv6 Source Address | IPv6 Home Agent address |  |  |
| IPv6 Destination Address | IPv6 Home Address configured by UE from Home Network Prefix assigned to UE during preamble |  | UE IPv4 |
|  | IPv6 CoA |  | UE IPv6-visited |
| Routing Header | IPv6 Home Address configured by the UE from Home Network Prefix assigned to the UE during preamble |  | UE IPv6-visited |
| B.R. Type | ‘00000001’B | B.R.I |  |
| Sequence Number | Set by the SS |  |  |
| Revocation Trigger | ‘00000001’B |  |  |
| P | ‘0’B |  |  |
| G | ‘0’B |  |  |
| V | ‘0’B |  |  |

| Condition | Explanation |
| --- | --- |
| UE IPv4 | UE is in an IPv4 visited network (see RFC 5555) |
| UE IPv6-visited | UE is in an IPv6 visited network (see RFC 5555) |

#### -	Binding Revocation Acknowledgement

This message is sent by the UE to the SS.

Table 4.7C.2-5: Binding Revocation Acknowledgement

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| IPv4 Header |  |  | UE IPv4 |
| IPv4 Source Address | IPv4 CoA |  | UE IPv4 |
| IPv4 Destination Address | UE IPv4 Home Agent Address |  | UE IPv4 |
| UDP header |  | UDP header | UE IPv4 |
| Source Port | ‘0001000001011111’B | Source Port | UE IPv4 |
| Destination port | Same as Source port in Binding Revocation | Destination port | UE IPv4 |
| IPv6 Header |  |  |  |
| IPv6 Source Address | IPv6 Home Address configured by UE |  | UE IPv4 |
|  | IPv6 CoA |  | UE IPv6-visited |
| IPv6 Destination Address | IPv6 of Home Agent address | IPv6 Destination Address |  |
| Destination Header | IPv6 Home Address configured by the UE |  | UE IPv6-visited |
| B.R. Type | ‘00000010’B | B.R.A |  |
| Sequence Number | Same value as Sequence Number sent by the SS in Binding Revocation |  |  |
| Status | ‘00000000’B | Success |  |
| P | '0’B |  |  |
| G | '0’B |  |  |
| V | '0’B |  |  |

| Condition | Explanation |
| --- | --- |
| UE IPv4 | UE is in an IPv4 visited network (see RFC 5555) |
| UE IPv6-visited | UE is in an IPv6 visited network (see RFC 5555) |

## 4.7D	Default GERAN message and information element contents

### 4.7D.1	GPRS message

#### –	PS HANDOVER COMMAND

Table 4.7D.1-1: PS HANDOVER COMMAND

| Derivation Path: 44.060 clause 11.2.43.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PAGE_MODE |  | Default |  |  |
| Global TFI |  | TFI of mobile station uplink TBF |  |  |
| CONTAINER_ID |  | 00 |  |  |
| PS Handover to A/Gb Mode Payload |  | 00 | PS Handover RR Info |  |
| PS Handover RR Info flag |  | 00 |  |  |
| PS Handover Radio Resources IE |  | Present |  |  |
| Handover Reference |  | 0 |  |  |
| ARFCN |  | default ARFCN for BCCH of GERAN cell |  |  |
| SI |  | 00 | Non-synchronized |  |
| NCI |  | 0 |  |  |
| BSIC |  | BSIC of GERAN cell |  |  |
| 0 <CCN_ACTIVE> |  | not present |  |  |
| 0 <3G_CCN_ACTIVE> |  | not present |  |  |
| 0 <CCN Support Description> |  | not present |  |  |
| Frequency Parameters |  | default PDTCH of GERAN cell |  |  |
| NETWORK_CONTROL_ORDER |  | 2 |  |  |
| 0 <Global Packet Timing Advance> |  | not present |  |  |
| EXTENDED_DYNAMIC_ALLOCATION |  | 0 |  |  |
| RLC_RESET |  | 1 | RLC is reset |  |
| 0 <PO> |  | not present |  |  |
| 0 <Uplink Control Timeslot> |  | not present |  |  |
| 0 GPRS mode |  | Uplink TBF assignment for default PDTCH of GERAN cell |  |  |
| 0 <NAS Container for PS Handover IE> |  | not present |  |  |

## 4.7E	Default HTTP messages for communication with the ProSe Function

#### -	HTTP Request

This message is sent by the UE to the SS.

Table 4.7E-1: HTTP Request

| Header/param | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| Request-Line |  |  |  |
| Method | “POST” |  |  |
| Request-URI | Request-URI |  |  |
| Version | “HTTP 1.1” |  |  |
| Content-Type |  |  |  |
| media-type | “application/3gpp-prose+xml” |  |  |
| Content-Length |  |  |  |
| value | length of the message body |  |  |
| Message-body | PC3 Control Protocol messages |  |  |
| Note: HTTP request may contain further headers which are ignored. |  |  |  |

#### -	HTTP Response

This message is sent by the SS to the UE.

Table 4.7E-2: HTTP Response

| Header/param | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| Status-Line |  |  |  |
| Version | “HTTP 1.1” |  |  |
| Code | 200 |  |  |
| Reason | “OK” |  |  |
| Content-Type |  |  |  |
| media-type | “application/3gpp-prose+xml” |  |  |
| Content-Length |  |  |  |
| value | length of the message body |  |  |
| Message-body | PC3 Control Protocol messages |  |  |

## 4.7F	Default ProSe messages

### 4.7F.1	ProSe discovery messages

#### -	DISCOVERY_REQUEST

This message is sent by the UE to the SS.

Table 4.7F.1-1: DISCOVERY_REQUEST

| Derivation path: 24.334 [57], clause 11.2.4.2. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| discovery-request[1] { |  |  |  |
| transaction-ID | Any allowed value |  |  |
| command | 2 | monitor |  |
|  | 1 | announce |  |
| UE-identity | IMSI |  |  |
| ProSe-Application-ID | px_ProSeMonApplicationIdentity1 |  |  |
| application-identity | Any allowed value |  |  |
| } |  |  |  |
| ... |  |  |  |
| discovery-request[N]{} | Any allowed value if present | A UE can include one or multiple transactions in one DISCOVERY_REQUEST message for one or more ProSe Application IDs |  |

#### -	DISCOVERY_RESPONSE

This message is sent by the SS to the UE.

Table 4.7F.1-2: DISCOVERY_RESPONSE

| Derivation path: 24.334 [57], clause 11.2.4.3. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Current-time | Current UTC time |  |  |
| Max-Offset | 32 | 32 seconds |  |
| response-announce[1] { |  |  |  |
| transaction-ID | Same as that included by UE in the relevant DISCOVERY_REQUEST message and the relevant discovery-request |  |  |
| ProSe Application Code[1] { |  | 184 bits See TS 23.003 [2], section 24.3. |  |
| PLMN ID { |  | 24 bits The PLMN ID of the ProSe Function that assigned the ProSe Application Code, i.e. Mobile Country Code (MCC) and Mobile Network Code (MNC). |  |
| Scope | '10' | "country-specific scope", i.e. only MNC is wild carded |  |
| Spare | '0' |  |  |
| E | '1' | For Rel-12 MCC and the MNC of the ProSe Function that has assigned the ProSe Application Code are included in the PLMN ID in ProSe Application Code |  |
| MCC | The MCC part of the The PLMN ID of the ProSe Function that assigned the ProSe Application Code (the relevant PLMN will be set in the TC) | fixed length of 10 bits and shall be coded as the binary representation of their decimal value |  |
| MNC | The MNC part of the The PLMN ID of the ProSe Function that assigned the ProSe Application Code (the relevant PLMN will be set in the TC) | fixed length of 10 bits and shall be coded as the binary representation of their decimal value |  |
| } |  |  |  |
| TemporaryID | 1111111100000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000011111111 | 160 bits - see setting of bit 'E' above. A temporary identity that corresponds to the ProSe Application ID Name. |  |
| } |  |  |  |
| validity-timer-T4000 | 5 | Indicating the time during which the Application Code is valid Integer in the 1-525600 range representing the timer value in unit of minutes |  |
| discovery-key | 1000100000001000 0100010000001000 0010001000001000 0001000100001000 0000100010001000 0000010001001000 0000001000101000 0000000100011000 | 128-bit As defined in TS 33.303 [60] and TS 33.220 [61] |  |
| } |  |  |  |
| response-monitor[1] { |  |  |  |
| transaction-ID | Same as that included by UE in the relevant DISCOVERY_REQUEST message and the relevant discovery-request |  |  |
| discovery-filter[1] { |  |  |  |
| ProSe Application Code { |  | 184 bits See TS 23.003 [2], section 24.3. |  |
| PLMN ID { |  | 24 bits The PLMN ID of the ProSe Function that assigned the ProSe Application Code, i.e. Mobile Country Code (MCC) and Mobile Network Code (MNC). |  |
| Scope | '10' | "country-specific scope", i.e. only MNC is wild carded |  |
| Spare | '0' |  |  |
| E | '1' | For Rel-12 MCC and the MNC of the ProSe Function that has assigned the ProSe Application Code are included in the PLMN ID in ProSe Application Code |  |
| MCC | The MCC part of the The PLMN ID of the ProSe Function that assigned the ProSe Application Code (the relevant PLMN will be set in the TC) | fixed length of 10 bits and shall be coded as the binary representation of their decimal value |  |
| MNC | The MNC part of the The PLMN ID of the ProSe Function that assigned the ProSe Application Code (the relevant PLMN will be set in the TC) | fixed length of 10 bits and shall be coded as the binary representation of their decimal value |  |
| } |  |  |  |
| TemporaryID | 1111111100000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000011111111 | 160 bits - see setting of bit 'E' above. A temporary identity that corresponds to the ProSe Application ID Name. |  |
| } |  |  |  |
| ProSe Application Mask[1] | 0000111111111100 00000000 1111111111111111 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 | 184 bits - The length of the ProSe Application Mask is the same as the length of ProSe Application Code Full matching for the MCC part is required, as well as for the first 16 bits of the Temporary ID in order a received ProSe Application Code to result in a match |  |
| ProSe Application Mask[2] | 0000111111111100 00000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 1111111111111111 | 184 bits - The length of the ProSe Application Mask is as same as the length of ProSe Application Code Full matching for the MCC part is required, as well as for the last 16 bits of the Temporary ID |  |
| TTLTimer T4002 | 5 | Indicating the time during which the filter is valid Integer in the 1-525600 range representing the timer value in unit of minutes |  |
| } |  |  |  |
| } |  |  |  |
| response-reject | Not Present |  |  |

#### -	MATCH_REPORT

This message is sent by the UE to the SS.

Table 4.7F.1-3: MATCH_REPORT

| Derivation path: 24.334 [57], clause 11.2.4.4. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| transaction-ID | a new transaction ID |  |  |
| ProSe-Application-Code | A code received in a PC5_DISCOVERY message for which there was a match event and for which the UE does not have an associated ProSe Application ID | The expected value will be set in the relevant TC. |  |
| UE-identity | IMSI |  |  |
| Monitored-PLMN-id | The monitored in the TC PLMN | The expected value will be set in the relevant TC. |  |
| VPLMN-id | Not included |  |  |
| MIC | MIC of the PC5_DISCOVERY message that contained the ProSe Application Code for which there was a match event |  |  |
| UTC-based Counter | - the 28 most significant bits of the UTC-based counter shall be set to the 28 most significant bits of the UTC time provided by the lower layers for the PC5_DISCOVERY message that contained the ProSe Application Code for which there was a match event encoded as specified in TS 24.334 [57], subclause 12.2.2.18; and - the 4 least significant bits of the UTC-based counter shall be set to the 4 least significant bits of the UTC-based counter contained in the PC5_DISCOVERY message that contained the ProSe Application Code for which there was a match event, as specified in 3GPP TS 33.303 [60] | The UTC is expressed in unit of seconds and coded in binary format as the 32 least significant bits of the Coordinated Universal Time as defined in 3GPP TS 36.331 [17] |  |
| metadata-flag | Any allowed value |  |  |

#### -	MATCH_REPORT_ACK

This message is sent by the SS to the UE.

Table 4.7F.1-4: MATCH_REPORT_ACK

| Derivation path: 24.334 [57], clause 11.2.4.5. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Current-Time | Current UTC time |  |  |
| match-ack { |  |  |  |
| transaction-ID | the transaction ID received in the MATCH_REPORT |  |  |
| ProSe-Application-ID |  | a ProSe Application ID which the UE does not have |  |
| validity-timer-T4004 | 4 | 4 min |  |
| metadata | Not Present |  |  |
| match-report-refresh-timer-T4006 | 1 | 1 min |  |
| } |  |  |  |

#### -	PC5_DISCOVERY

This message is sent by the UE or by the SS-UE.

Table 4.7F.1-5: PC5_DISCOVERY for open ProSe direct discovery

| Derivation path: 24.334 [57], Table 11.2.5.1.1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ProSe Application Code { |  |  |  |
| PLMN ID | Same as the PLMN ID provided in the DISCOVERY_RESPONSE message sent during the latest Monitor request procedure |  |  |
| TemporaryID | Same as the TemporaryID provided in the DISCOVERY_RESPONSE message sent during the latest Monitor request procedure |  |  |
| } |  |  |  |
| MIC | Computed by UE based on Message Type, discovery-key and ProSe Application Code included by SS in DISCOVERY_RESPONSE sent during the latest Monitor request procedure, and UTC-based counter associated with the discovery slot. See TS 33.303 [60], A.2. The 32 least significant bits of the output of the KDF, TS 33.220 [61], Annex B. |  |  |
| UTC-based Counter LSB | four least significant bits of the UTC-based counter associated with the discovery slot | The UTC is expressed in unit of seconds and coded in binary format as the 32 least significant bits of the Coordinated Universal Time as defined in 3GPP TS 36.331 [17] |  |

Table 4.7F.1-5A: PC5_DISCOVERY for Group Member Discovery Announcement

| Derivation path: 24.334 [57], Table 11.2.5.1.7. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Message Type { |  |  |  |
| Discovery type | '01'B | "Restricted discovery" |  |
| Content type | '0110' | "Group Member Discovery Announcement or Group Member Discovery Response" |  |
| Discovery model | '01' | "Model A" |  |
| } |  |  |  |
| Discovery Group ID | The configured Discovery Group ID parameter identifying the discovery group to be announced | 24-bit long bit string |  |
| User Info ID | The configured User Info ID for the group member discovery parameter | Announcer Info 48-bit long bit string |  |
| ProSe UE ID | The configured Layer 2 ID used for unicast communication | 24-bit long bit string |  |
| Spare |  | The length of this parameter is variable and is indicated in a message where this parameter is included |  |
| MIC | Computed in accordance with TS 33.303 [60], A.2. |  |  |
| UTC-based Counter LSB |  | Set to the eight least significant bits of the UTC-based counter The UTC is expressed in unit of seconds and coded in binary format as the 32 least significant bits of the Coordinated Universal Time as defined in 3GPP TS 36.331 [17] |  |

Table 4.7F.1-5B: PC5_DISCOVERY for Group Member Discovery Solicitation

| Derivation path: 24.334 [57], Table 11.2.5.1.8. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Message Type { |  |  |  |
| Discovery type | '01'B | "Restricted discovery" |  |
| Content type | '0111' | "Group Member Discovery Solicitation" |  |
| Discovery model | '10' | "Model B" |  |
| } |  |  |  |
| Discovery Group ID | The configured Discovery Group ID parameter identifying the discovery group to be solicited | 24-bit long bit string |  |
| User Info ID | The configured User Info ID for the group member discovery parameter | Discoverer Info 48-bit long bit string |  |
| GMDS Composition { |  |  |  |
| TUII | Any value |  |  |
| TGII | Any value |  |  |
| } |  |  |  |
| Target User Info | IF TUII='1' THEN the User Info ID of the targeted discoveree user - 48-bit long bit string ELSE Not present |  |  |
| Target Group Info | IF TGII='1' THEN ProSe Layer 2 Group ID - 24-bit long bit string ELSE Not present |  |  |
| Spare |  | The length of this parameter is variable and is indicated in a message where this parameter is included |  |
| MIC | Computed in accordance with TS 33.303 [60], A.2. |  |  |
| UTC-based Counter LSB |  | Set to the eight least significant bits of the UTC-based counter The UTC is expressed in unit of seconds and coded in binary format as the 32 least significant bits of the Coordinated Universal Time as defined in 3GPP TS 36.331 [17] |  |

Table 4.7F.1-5C: PC5_DISCOVERY for Group Member Discovery Response

| Derivation path: 24.334 [57], Table 11.2.5.1.9. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Message Type { |  |  |  |
| Discovery type | '01'B | "Restricted discovery" |  |
| Content type | '0110' | "Group Member Discovery Announcement or Group Member Discovery Response" |  |
| Discovery model | '10' | "Model B" |  |
| } |  |  |  |
| Discovery Group ID | The received Discovery Group ID parameter of the PC5_DISCOVERY message for Group Member Discovery Solicitation | 24-bit long bit string |  |
| User Info ID | The configured User Info ID for the group member discovery parameter | Discoveree Info 48-bit long bit string |  |
| ProSe UE ID | The configured Layer 2 ID used for unicast communication | 24-bit long bit string |  |
| Spare |  | The length of this parameter is variable and is indicated in a message where this parameter is included |  |
| MIC | Computed in accordance with TS 33.303 [60], A.2. |  |  |
| UTC-based Counter LSB |  | Set to the eight least significant bits of the UTC-based counter The UTC is expressed in unit of seconds and coded in binary format as the 32 least significant bits of the Coordinated Universal Time as defined in 3GPP TS 36.331 [17] |  |

### 4.7F.2	Messages transmitted over the PC3ch interface

#### -	USAGE_INFORMATION_REPORT_LIST

This message is sent by the UE to the SS.

Table 4.7F.2-1: USAGE_INFORMATION_REPORT_LIST

| Derivation path: 24.334 [57], clause 11.3.4.2. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| transaction-ID | Any allowed value |  |  |
| UE-identity | IMSI |  |  |
| usage-information-report[1] { | 1 Report |  |  |
| sequence-number | 1 | First report after power up |  |
| coverage { |  |  |  |
| in-coverage | FALSE | The UE was out of E-UTRAN coverage |  |
| timestamp | Not Present or Any allowed Value |  |  |
| location[1] | Not Present | UE was out of coverage i.e. the UE was not camping on nor the UE was in the EMM-CONNECTED mode on any E-UTRA cell |  |
| } |  |  |  |
| group[1] { |  | The UE is configured to report the group parameters, see EFPROSE_UIRC |  |
| prose-layer2-group-ID | As preconfigured in EFPROSE_POLICY |  |  |
| prose-group-IP-multicast-address | As preconfigured in EFPROSE_POLICY |  |  |
| first-transmission-timestamp | Any allowed Value |  |  |
| first-reception-timestamp | Any allowed Value |  |  |
| UE-source-IP-address | Any allowed Value |  |  |
| prose-UE-id | As preconfigured in EFPROSE_POLICY |  |  |
| transmitter { |  |  |  |
| source-IP-address | Any allowed Value |  |  |
| prose-UE-id | Any allowed value |  |  |
| } |  |  |  |
| transmission { |  |  |  |
| in-coverage | FALSE |  |  |
| ECGI | Not Present |  |  |
| amount | Any allowed Value |  |  |
| timestamp | Any allowed Value |  |  |
| radio-resources-ind | 2 | the configured radio resources were used |  |
| radio-frequency | carrierFreq-r12, Table 6.8.1.1-1 | the configured radio frequency |  |
| } |  |  |  |
| reception { |  |  |  |
| in-coverage | FALSE |  |  |
| ECGI | Not Present |  |  |
| amount | Any allowed Value |  |  |
| timestamp | Any allowed Value |  |  |
| radio-resources-ind | 2 | the configured radio resources |  |
| radio-frequency | carrierFreq-r12, Table 6.8.1.1-1 | the configured radio frequency |  |
| } |  |  |  |
| ..} |  |  |  |
| radio-parameter-set { | 2 entries in any order refering to SL-Preconfiguration-r12, Table 6.8.1.1-1 SLCONF1 and SLCONF2 |  |  |
| { | Entry 1 |  |  |
| params | the radio parameters used for ProSe direct communication, see SL-Preconfiguration-r12, Table 6.8.1.1-1 cond SLCONF1 |  |  |
| timestamp | Any allowed Value |  |  |
| } |  |  |  |
| { | Entry 2 |  |  |
| params | the radio parameters used for ProSe direct communication, see SL-Preconfiguration-r12, Table 6.8.1.1-1 cond SLCONF2 |  |  |
| timestamp | Any allowed Value |  |  |
| } |  |  |  |
| } |  |  |  |
| app-specific-info | Any allowed Value |  |  |

#### -	USAGE_INFORMATION_REPORT_LIST_RESPONSE

This message is sent by the SS to the UE.

Table 4.7F.2-2: USAGE_INFORMATION_REPORT_LIST_RESPONSE

| Derivation path: 24.334 [57], clause 11.3.4.3. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| response-accept { |  |  |  |
| transaction-ID | The value of the transaction ID of the related USAGE_INFORMATION_REPORT_LIST message |  |  |
| } |  |  |  |
| response-reject { | Not present |  |  |
| transaction-ID |  |  |  |
| cause-value |  |  |  |
| } |  |  |  |

### 4.7F.3	ProSe Direct Communication Messages

#### -	DIRECT_COMMUNICATION_ACCEPT

This message can be sent by the UE or the SS.

Table 4.7F.3-0A: DIRECT_COMMUNICATION_ACCEPT

| Derivation path: 24.334 [57], clause 11.4.3.1.1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Sequence Number | 0 | incremented for each outgoing new PC5 Signalling message |  |
| IP Address Config | Any allowed value |  |  |
| Link Local IPv6 Address | Present if the IP Address Config IE is set to 'address allocation not support' Not Present otherwise | 128-bit IPv6 address |  |

#### -	DIRECT_COMMUNICATION_KEEPALIVE

This message can be sent by the UE or the SS.

Table 4.7F.3-0B: DIRECT_COMMUNICATION_KEEPALIVE

| Derivation path: 24.334 [57], clause 11.4.5.1.1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Sequence Number | The Sequence number used in the previously sent PC5 Signalling message + 1 |  |  |
| Keepalive Counter | 0 if this is the first keepalive message being sent otherwise - the value sent in the preceeding acknowledged DIRECT_COMMUNICATION_KEEPALIVE message + 1,or, - the same value if this is a retransmission |  |  |
| Maximum Inactivity Period | Any allowed value or Not present |  |  |

#### -	DIRECT_COMMUNICATION_KEEPALIVE_ACK

This message can be sent by the UE or the SS.

Table 4.7F.3-0C: DIRECT_COMMUNICATION_KEEPALIVE_ACK

| Derivation path: 24.334 [57], clause 11.4.6.1.1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Sequence Number | The Sequence number used in the previously sent PC5 Signalling message + 1 |  |  |
| Keepalive Counter | the same value as the one received in the preceeding DIRECT_COMMUNICATION_KEEPALIVE message |  |  |

#### -	DIRECT_COMMUNICATION_RELEASE

This message can be sent by the UE or the SS.

Table 4.7F.3-0D: DIRECT_COMMUNICATION_RELEASE

| Derivation path: 24.334 [57], clause 11.4.7.1.1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Sequence Number | The Sequence number used in the previously sent PC5 Signalling message + 1 |  |  |
| Release Reason | Any allowed value |  |  |

#### -	DIRECT_COMMUNICATION_RELEASE_ACCEPT

This message can be sent by the UE or the SS.

Table 4.7F.3-0E: DIRECT_COMMUNICATION_RELEASE_ACCEPT

| Derivation path: 24.334 [57], clause 11.4.8.1.1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Sequence Number | The Sequence number used in the previously sent PC5 Signalling message + 1 |  |  |

#### -	DIRECT_COMMUNICATION_REQUEST

This message can be sent by the UE or the SS.

Table 4.7F.3-0F: DIRECT_COMMUNICATION_REQUEST

| Derivation path: 24.334 [57], clause 11.4.2.1.1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Sequence Number | 0 | incremented for each outgoing new PC5 Signalling message |  |
| User Info { |  |  |  |
| Type of User Info | Any allowed value |  |  |
| Odd/even indication | Any allowed value | Reflecting the number of identity digits |  |
| Identity digits | Any allowed value |  |  |
| } |  |  |  |
| IP Address Config | Any allowed value |  |  |
| Maximum Inactivity Period | Any allowed value |  |  |
| Nonce_1 | Any allowed value | 128-bit nonce value generated by the UE which initiated the procedure |  |
| UE Security Capabilities | Any allowed value |  |  |
| MSB of KD-sess ID | Any allowed value | the 8 most significant bits of the KD-sess ID |  |
| KD ID | Any allowed value |  |  |
| Relay Service Code | Not present |  |  |
| Signature | Any allowed value | ECCSI signature content structure is specified in 3GPP TS 33.303 [60] IETF RFC 6507 |  |
| Link Local IPv6 Address | Present if the IP Address Config IE is set to 'address allocation not support' Not Present otherwise | 128-bit IPv6 address |  |

#### -	DIRECT_SECURITY_MODE_COMMAND

This message can be sent by the UE or the SS.

Table 4.7F.3-0G: DIRECT_SECURITY_MODE_COMMAND

| Derivation path: 24.334 [57], clause 11.4.12.1.1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Sequence Number | The Sequence number used in the previously sent PC5 Signalling message + 1 |  |  |
| UE Security Capabilities | Any allowed value |  |  |
| Nonce 2 | Any allowed value | 128-bit nonce value generated by the UE which initiated the procedure |  |
| Chosen Algorithms | Any allowed value |  |  |
| LSB of KD-sess ID | Any allowed value | the 8 least significant bits of the KD-sess ID. |  |
| MSB of KD ID | Any allowed value or Not included | the 8 most significant bits of the KD-sess ID |  |
| KD Freshness | Any allowed value or Not included | 128-bit nonce value generated by initiating PKMF to ensure that any calculated KD is fresh |  |
| GPI | Any allowed value or Not included | the GBA Push Information as defined in 3GPP TS 33.223 |  |
| User Info { |  |  |  |
| Type of User Info | Any allowed value |  |  |
| Odd/even indication | Any allowed value | Reflecting the number of identity digits |  |
| Identity digits | Any allowed value |  |  |
| } |  |  |  |
| Signature | Any allowed value | ECCSI signature content structure is specified in 3GPP TS 33.303 [60] IETF RFC 6507 |  |
| Encrypted Payload | Any allowed value | encrypted data encapsulating the shared secret key to be used for the established link The exact content structure is specified in 3GPP TS 33.303 [60] |  |

#### -	DIRECT_SECURITY_MODE_COMPLETE

This message can be sent by the UE or the SS.

Table 4.7F.3-0H: DIRECT_SECURITY_MODE_COMPLETE

| Derivation path: 24.334 [57], clause 11.4.13.1. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Sequence Number | The Sequence number used in the previously sent PC5 Signalling message + 1 |  |  |
| LSB of KD ID | Any allowed value or Not present | Optional 16 least significant bits of KD ID |  |

#### -	KEY_REQUEST

This message is sent by the UE to the SS.

Table 4.7F.3-1: KEY_REQUEST

| Derivation path: 33.303 [60], clause E.4.2. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| transaction-ID | Any allowed value |  |  |
| AlgorithmAvailable | Any allowed value | Encryption algorithm the UE supports for one-to-many communications |  |
| GroupKeyReq { | One entry |  |  |
| GroupId | Any allowed value | Group that the UE is requesting keys for (Range 0-167777215) |  |
| PGKId | 0 | UE does not have any PGKs for this group |  |
| anyExt | Not Present |  |  |
| } |  |  |  |
| PRUKId | Not Present |  |  |
| KDRequest | Not Present |  |  |
| Key Exchange Payload | Not Present |  |  |
| GroupKeyStop | Not Present |  |  |
| PSDiscoverySecurityCapabilities | Not Present |  |  |
| RelayDiscoveryKeyRequest | Not Present |  |  |
| RelayDiscoveryKeyStop | Not Present |  |  |
| GroupMemberDiscoveryKeyRequest | Not Present |  |  |
| GroupMemberDiscoveryKeyStop | Not Present |  |  |
| anyExt | Not Present |  |  |

#### -	KEY_RESPONSE

This message is sent by the SS to the UE.

Table 4.7F.3-2: KEY_RESPONSE

| Derivation path: 33.303 [60], clause E.4.3. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| transaction-ID | Same as that received in KEY_REQUEST |  |  |
| GroupNotSupported | Not Present |  |  |
| GroupResponse { | One entry |  |  |
| GroupId | Same as that received in KEY_REQUEST |  |  |
| GroupMemberID | 1 |  |  |
| AlgorithmInfo | ‘001’B | EPS encryption algorithm 128-EEA1 |  |
| } |  |  |  |
| Key-info { | One entry |  |  |
| PMK-ID | ‘0000000000000001’H |  |  |
| PMK | ‘00000000000000000000000000000001’H |  |  |
| } |  |  |  |
| PRUKinfo | Not Present |  |  |
| PRUKError | Not Present |  |  |
| KDResponse | Not Present |  |  |
| KDError | Not Present |  |  |
| RelayNotSupported | Not Present |  |  |
| RelayResponse | Not Present |  |  |
| GroupMemberDiscoveryNotSupported | Not Present |  |  |
| GroupMemberDiscoveryResponse | Not Present |  |  |
| CurrentTime | Not Present |  |  |
| MaxOffset | Not Present |  |  |
| anyExt | Not Present |  |  |

#### -	MIKEY Key Delivery Message

This message is sent by the SS to the UE.

Table 4.7F.3-3: MIKEY Key Delivery Message

| Derivation path: 33.303 [60], clause D.3. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MIKEY Common Header { |  |  |  |
| version | ‘00000001’B |  |  |
| data type | ‘00000000’B | Pre-shared key message |  |
| Next payload | ‘00000101’B | Next payload is timestamp |  |
| V | ‘1’B | Response expected |  |
| PRF func | ‘0000000’B |  |  |
| CSB ID | Random number | Any 32 bit random number |  |
| #CS | ‘00000000’B |  |  |
| CS ID map type | ‘00000001’B | Empty map |  |
| CS ID map Info | FFS |  |  |
| } |  |  |  |
| Timestamp Payload { |  |  |  |
| Next payload | ‘00001011’B | Next payload is RAND |  |
| TS Type | ‘00000010’B | Value 2 corresponds to Counter |  |
| TS Value | ‘00000000000000000000000000000001’B | Replay counter. |  |
| } |  |  |  |
| RAND Payload { |  |  |  |
| Next payload | ‘00000110’B | Next payload is ID |  |
| RAND len | ‘00010000’B | 16 Bytes RAND |  |
| RAND | 128-bit random number |  |  |
| } |  |  |  |
| IDi payload { |  |  |  |
| Next payload | ‘00000110’B | Next payload is ID |  |
| ID Type | 0 |  |  |
| ID len | Length of ID Data |  |  |
| ID data | Group Identity ¦¦ PGK ID @ FQDN of the ProSe Key Management Function |  |  |
| } |  |  |  |
| IDr payload { |  |  |  |
| Next payload | ‘00000001’B | Next payload is KEMAC |  |
| ID Type | 0 |  |  |
| ID len | Length of ID Data |  |  |
| ID data | PMK identity of the PMK used to protect the MIKEY message @ the FQDN of the ProSe Key Management Function |  |  |
| } |  |  |  |
| KEMAC Payload { |  |  |  |
| Next payload | ‘00000000’B | This is the last payload |  |
| Encr alg | ‘00000001’B | AES-CM-128 |  |
| Encr data len | Length of Encr Data |  |  |
| Encr Data { |  | Key Data payload is encrypted and carried |  |
| Next payload | ‘00000000’B | This is the last payload |  |
| Type | ‘0000’B | TEK |  |
| KV | ‘0010’B | The key has a start and expiration time |  |
| Key data len | ‘0000000010000000’B | 128 bits |  |
| Key data | ‘0000 0000 0000 0000 0000 0000 0000 0001’[OCT] | PGK |  |
| KV data { |  |  |  |
| VF length | FFS |  |  |
| Valid from | FFS |  |  |
| VT length | FFS |  |  |
| Valid to | FFS |  |  |
| } |  |  |  |
| } |  |  |  |
| Mac alg | ‘00000001’B | HMAC-SHA-1-160 |  |
| MAC | Message Authentication Code of the message |  |  |
| } |  |  |  |

#### -	MIKEY Verification Message

This message is sent by the UE to the SS.

Table 4.7F.3-4: MIKEY Verification Message

| Derivation path: 33.303 [60], clause D.3. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MIKEY Common Header { |  |  |  |
| version | ‘00000001’B |  |  |
| data type | ‘00000000’B | Pre-shared key message |  |
| Next payload | ‘00000101’B | Next payload is timestamp |  |
| V | ‘0’B |  |  |
| PRF func | ‘0000001’B |  |  |
| CSB ID | Same as sent by ProSe Function in the MIKEY message carrying PGK |  |  |
| #CS | ‘00000000’B |  |  |
| CS ID map type | ‘00000001’B | Empty map |  |
| CS ID map Info | FFS |  |  |
| } |  |  |  |
| Timestamp Payload { |  |  |  |
| Next payload | ‘00001011’B | Next payload is RAND |  |
| TS Type | ‘00000010’B | Value 2 corresponds to Counter |  |
| TS Value | ‘00000000000000000000000000000001’B | Replay counter. |  |
| } |  |  |  |
| IDr payload { |  |  |  |
| Next payload | ‘00001001’B | Next payload is Verification Payload |  |
| ID Type | 0 |  |  |
| ID len | Length of ID data |  |  |
| ID data | PMK identity of the PMK used to protect the MIKEY message @ the FQDN of the ProSe Key Management Function |  |  |
| } |  |  |  |
| Verification Message Payload { |  |  |  |
| Next payload | ‘00000000’B | This is the last payload |  |
| Auth alg | ‘00000001’B | AES-CM-128 |  |
| Verification data | Message Authentication Code |  |  |
| } |  |  |  |

## 4.7G	Default IKEv2 message and information element contents

#### -	IKE_SA_INIT request

This message is sent by the UE to the SS.

Table 4.7G-1: IKE_SA_INIT request

| Derivation path: IETF RFC 5996 [57] |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| IKE Header |  |  |  |
| Initiator’s IKE_SA SPI | Not checked |  |  |
| Responder’s IKE_SA SPI | 0 | First message in IKE_SA_INIT exchange |  |
| Next Payload | ‘00100001’B | SA |  |
| Exchange Type | ‘00100010’B | IKE_SA_INIT |  |
| Security Association Payload |  | The presence of all Transform type and related Transform ID are checked. These are allowed in one or more Proposals in any combination. |  |
| Next Payload | ’00100010’B | KE |  |
| More proposal | Not checked |  |  |
| Proposal # | Not checked | Cryptographic suite |  |
| Protocol ID | ‘00000001’B | IKE |  |
| SPI size | Not checked |  |  |
| Number of transforms | Not checked |  |  |
| More transform or Last transform | Not checked |  |  |
| Transform type | ‘00000001’B | Encryption Algorithm |  |
| Transform ID | ‘00000011’B | ENCR_3DES |  |
| More transform or Last transform | Not checked |  |  |
| Transform type | ‘00000001’B | Encryption Algorithm |  |
| Transform ID | ‘00001100’B | ENCR_AES_CBC |  |
| More transform or Last transform | Not checked |  |  |
| Transform type | ‘00000010’B | Pseudorandom Function |  |
| Transform ID | ‘00000010’B | PRF_HMAC_SHA1 |  |
| More transform or Last transform | Not checked |  |  |
| Transform type | ‘00000011’B | Integrity Algorithm |  |
| Transform ID | ‘00000010’B | AUTH_HMAC_SHA1_96 |  |
| More transform or Last transform | Not checked |  |  |
| Transform type | ‘00000011’B | Integrity Algorithm |  |
| Transform ID | ‘00000101’B | AUTH_AES_XCBC_96 |  |
| More transform or Last transform | Not checked |  |  |
| Transform type | ‘00000100’B | Diffie-Hellman group |  |
| Transform ID | ‘00000010’B | 1024-bit MODP |  |
| Last transform or More transform | Not checked |  |  |
| Transform type | ‘00000100’B | Diffie-Hellman group |  |
| Transform ID | ‘00001110’B | 2048-bit MODP |  |
| Key Exchange Payload |  |  |  |
| Next Payload | ‘00101000’B | Nonce |  |
| Diffie-Hellman Group Num | ‘0000000000000010’B or ‘0000000000001110’B | DH group 2 or 14 |  |
| Key Exchange data | Not checked |  |  |
| Nonce Payload |  |  |  |
| Next Payload | ‘00000000’B | No Next Payload if Nonce is the last payload |  |
| Nonce data | Not checked |  |  |
| NOTE 1: The order of Payloads/fields is not checked, unless explicitly specified. Additional Payloads/fields are ignored. NOTE 2: The Check for Next Payload is to check the presence of (SA, KE, Nonce and No Next Payload). The SS shall not reject as invalid a message with those payloads in any other order. |  |  |  |

#### -	IKE_SA_INIT response

This message is sent by the SS to the UE.

Table 4.7G-2: IKE_SA_INIT response

| Derivation path: IETF RFC 5996 [57] |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| IKE Header |  |  |  |
| Initiator’s IKE_SA SPI | Same as that set by the UE in IKE_SA_INIT request |  |  |
| Responder’s IKE_SA SPI | Set by the SS |  |  |
| Next Payload | ‘00100001’B | SA |  |
| Exchange Type | ‘00100010’B | IKE_SA_INIT |  |
| Security Association Payload |  |  |  |
| Next Payload | ’00100010’B | KE |  |
| Proposal | One of the proposals included in IKE_SA_INIT request |  |  |
| Key Exchange Payload |  |  |  |
| Next Payload | ‘00101000’B | Nonce |  |
| Diffie-Hellman Group Num | One of the proposals included in IKE_SA_INIT request |  |  |
| Key Exchange data | Set by the SS |  |  |
| Nonce Payload |  |  |  |
| Next Payload | ‘00000000’B | No Next Payload if Nonce is the last payload |  |
| Nonce data | Set by the SS |  |  |

#### -	IKE_AUTH_request

This message is sent by the UE to the SS.

Table 4.7G-3: IKE_AUTH request

| Derivation path: IETF RFC 5996 [57] |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| IKE Header |  |  |  |
| Next Payload | ‘00101111’B or ‘00110000’B or ‘00100111’B | CP or EAP or AUTH |  |
| Exchange Type | ‘00100011’B | IKE_AUTH |  |
| NOTE 1: The order of Payloads/fields is not checked, unless explicitly specified. Additional Payloads/fields are ignored. |  |  |  |

#### -	IKE_AUTH response

This message is sent by the SS to the UE.

Table 4.7G-4: IKE_AUTH response

| Derivation path: IETF RFC 5996 [57] |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
|  |  | The presence and values for applicable fields are provided according to the test model [see TS 36.523-3 [20] cl. 4.4.6]. |  |

## 4.7H	Default TLS message and information element contents

NOTE:	Non handshake protocol details are ignored.

#### -	ClientHello

This message is sent by the UE to the SS.

Table 4.7H-1: ClientHello

| Field | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| Protocolversion | At least TLS 1.1 | TLS protocol versions 1.1 is defined in RFCs 4346 [62]. |  |
| Random | Not checked |  |  |
| SessionID | Empty |  |  |
| CipherSuite | A list of supported cryptographic options including one or more PSK-based ciphersuites |  |  |
| CompressionMethod | A list including CompressionMethod.null |  |  |

#### -	ServerHello

This message is sent by the SS to the UE.

Table 4.7H-2: ServerHello

| Field | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| Protocolversion | TLS 1.1 |  |  |
| Random | A SS generated random structure different from ClientHello |  |  |
| SessionID | Identity of the TLS session generated by SS |  |  |
| CipherSuite | One of the PSK-based ciphersuites listed in ClientHello | Ciphersuite according to PIXIT parameter (see TS 36.523-3 [20] cl. 9) |  |
| CompressionMethod | CompressionMethod.null |  |  |

#### -	ServerKeyExchange

This message is sent by the SS to the UE.

Table 4.7H-3: ServerKeyExchange

| Field | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| psk identity hint | “3GPP-bootstrapping” |  |  |

#### -	ServerHelloDone

This message is sent by the SS to the UE.

Table 4.7H-4: ServerHelloDone

| Field | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| Note: No fields present. |  |  |  |

#### -	ClientKeyExchange

This message is sent by the UE to the SS.

Table 4.7H-5: ClientKeyExchange

| Field | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| psk identity | Concatenation of “3GPP-bootstrapping”, “.” and B-TID |  |  |

#### -	ChangeCipherSpec

This message is sent by the UE or the SS.

Table 4.7H-6: ChangeCipherSpec

| Field | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| Enum | 1 |  |  |

#### -	Finished

This message is sent by the UE or the SS.

Table 4.7H-7: Finished

| Field | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| opaque | Not checked |  | UE |
| opaque | SS generated verify_data | The verify data contains the hash of the handshake messages. For details, see the RFC for TLS defined in annex E of 3GPP TS 33.310 [63]. | SS |

| Condition | Explanation |
| --- | --- |
| UE | Sent by the UE. |
| SS | Sent by the SS. |

## 4.7I	Default AT Command message and information element

#### -	AT Command +CATM

This AT Command is sent by the SS to the UE.

Table 4.7I-1: +CATM

| Derivation Path: 27.007 clause 15 |  |  |  |
| --- | --- | --- | --- |
| Field |  | Value/remark | Comment | Condition |
| +CATM= |  |  |  |  |
| <status> |  | 0 |  | Activation |
| <status> |  | 1 |  | Deactivation |

| Condition | Explanation |
| --- | --- |
| Activation | Activate UE test loop mode. |
| Deactivation | Deactivate UE test loop mode |

#### -	AT Command +CCUTLE

This AT Command is sent by the SS to the UE.

Table 4.7I-2: +CCUTLE

| Derivation Path: 27.007 clause 15 |  |  |  |
| --- | --- | --- | --- |
| Field |  | Value/remark | Comment | Condition |
| + CCUTLE = |  |  |  |  |
| <status> |  | 1 |  | Open |
| <status> |  | 0 |  | Close |
| <direction> |  | 1 |  | Transmit |
| <direction> |  | 0 |  | Receive |
| <format> |  | 1 |  | Receive |
| <length> |  | 1 |  | Receive |
| <monitor_list> |  | HEX ‘0000AA’ |  | Receive |

| Condition | Explanation |
| --- | --- |
| Open | Open UE test loop back mode E. |
| Close | Close UE test loop back mode E |
| Transmit | UE is configured to transmit V2X communication packets |
| Receive | UE is configured to receive V2X communication packets |

#### -	AT Command +CUTCR

This AT Command is sent by the SS to the UE.

Table 4.7I-3: +CUTCR

| Derivation Path: 27.007 clause 15 |  |  |  |
| --- | --- | --- | --- |
| Field |  | Value/remark | Comment | Condition |
| +CUTCR |  |  | This AT command has no additional fields |  |

#### -	AT Command +CUSPCREQ

This AT Command is sent by the SS to the UE.

Table 4.7I-4: +CUSPCREQ

| Derivation Path: 27.007 clause 15 |  |  |  |
| --- | --- | --- | --- |
| Field |  | Value/remark | Comment | Condition |
| +CUSPCREQ |  |  | This AT command has no additional fields |  |

## 4.8	Reference radio bearer configurations

### 4.8.1	General

### 4.8.2	SRB and DRB parameters and combinations

#### 4.8.2.1	SRB and DRB parameters

##### 4.8.2.1.1	SRB configurations

Table 4.8.2.1.1-1: SRB-ToAddMod-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SRB-ToAddMod-DEFAULT ::= SEQUENCE { |  |  |  |  |
| srb-Identity |  | 1 |  | SRB1 |
|  |  | 2 |  | SRB2 |
| rlc-Config CHOICE { |  |  |  |  |
| defaultValue |  |  |  |  |
| } |  |  |  |  |
| logicalChannelConfig CHOICE { |  |  |  |  |
| defaultValue |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 4.8.2.1.1-2: SRB4-ToAddMod-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SRB-ToAddMod-DEFAULT ::= SEQUENCE { |  |  |  |  |
| Srb-Identity |  | 2 |  |  |
| rlc-Config CHOICE { |  |  |  |  |
| am SEQUENCE { |  |  |  |  |
| ul-AM-RLC SEQUENCE { |  |  |  |  |
| t-PollRetransmit |  | ms45 |  |  |
| pollPDU |  | infinity |  |  |
| pollByte |  | infinity |  |  |
| maxRetxThreshold |  | t4 |  |  |
| } |  |  |  |  |
| dl-AM-RLC SEQUENCE { |  |  |  |  |
| t-Reordering |  | ms35 |  |  |
| t-StatusProhibit |  | ms0 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| logicalChannelConfig CHOICE { |  |  |  |  |
| explicityValue ::= SEQUENCE { |  |  |  |  |
| ul-SpecificParameters SEQUENCE { |  |  |  |  |
| priority |  | 15 |  |  |
| prioritisedBitRate |  | infinity |  |  |
| bucketSizeDuration |  | ms50 |  |  |
| logicalChannelGroup |  | 0 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| srb-Identity-v1530 |  | 4 |  |  |
| } |  |  |  |  |

##### 4.8.2.1.2	DRB PDCP configurations

4.8.2.1.2.1	DRB PDCP configurations for UM RLC

Table 4.8.2.1.2.1-1: PDCP-Config-DRB-UM

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PDCP-Config-DRB-UM ::= SEQUENCE { |  |  |  |  |
| discardTimer |  | ms100 | Suitable for real time services |  |
| rlc-AM SEQUENCE {} |  | Not present |  |  |
| rlc-UM SEQUENCE { |  |  |  |  |
| pdcp-SN-Size |  | Len12bits |  |  |
| } |  |  |  |  |
| headerCompression CHOICE { |  |  |  |  |
| notUsed |  | NULL |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

4.8.2.1.2.2	DRB PDCP configurations for AM RLC

Table 4.8.2.1.2.2-1: PDCP-Config-DRB-AM

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCP-Config-DRB-AM ::= SEQUENCE { |  |  |  |
| discardTimer | Infinity |  |  |
| rlc-AM SEQUENCE { |  |  |  |
| statusReportRequired | TRUE |  |  |
| } |  |  |  |
| rlc-UM SEQUENCE {} | Not present |  |  |
| headerCompression CHOICE { |  |  |  |
| notUsed | NULL |  |  |
| } |  |  |  |
| ul-DataSplitDRB-ViaSCG-r12 | TRUE | Uplink transmission of split DRB PDCP SDUs configured to be transmitted on the PSCell | DC_Setup_Split_DRB |
| t-Reordering-r12 | ms200 | ENUMERATED {ms0, ms20, ms40, ms60, ms80, ms100, ms120, ms140, ms160, ms180, ms200, ms220, ms240, ms260, ms280, ms300, ms500, ms750} | DC_Setup_Split_DRB |
| uplinkDataCompression-r15 SEQUENCE { |  |  | UDC |
| bufferSize-r15 | kbyte2 | ENUMERATED {kbyte2, kbyte4, kbyte8, spare1} |  |
| dictionary-r15 | Not present | ENUMERATED {sip-SDP, operator} |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| DC_Setup_Split_DRB | Setup of Split DRB |
| UDC | Setup of DRB with uplinkDataCompression |

##### 4.8.2.1.3	DRB RLC configurations

4.8.2.1.3.1	DRB UM RLC configurations

Table 4.8.2.1.3.1-1: RLC-Config-DRB-UM

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RLC-Config-DRB-UM ::= CHOICE { |  |  |  |  |
| um-Bi-Directional SEQUENCE { |  |  |  |  |
| ul-UM-RLC SEQUENCE { |  |  |  |  |
| sn-FieldLength |  | Size10 |  |  |
| } |  |  |  |  |
| dl-UM-RLC SEQUENCE { |  |  |  |  |
| sn-FieldLength |  | Size10 |  |  |
| t-Reordering |  | ms50 | ms50 provides sufficient margin |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

4.8.2.1.3.2	DRB AM RLC configurations

Table 4.8.2.1.3.2-1: RLC-Config-DRB-AM

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RLC-Config-DRB-AM ::= CHOICE { |  |  |  |  |
| am SEQUENCE { |  |  |  |  |
| ul-AM-RLC SEQUENCE { |  |  |  |  |
| t-PollRetransmit |  | ms80 |  |  |
| pollPDU |  | p128 |  |  |
| pollByte |  | kB125 |  |  |
| maxRetxThreshold |  | t4 |  |  |
| } |  |  |  |  |
| dl-AM-RLC SEQUENCE { |  |  |  |  |
| t-Reordering |  | ms80 | Is sufficient for 5 HARQ transmissions |  |
| t-StatusProhibit |  | Ms60 | Should be equal to or smaller than t-PollRetransmit. |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

##### 4.8.2.1.4	DRB Logical Channel configurations

Table 4.8.2.1.4-1: LogicalChannelConfig-DRB

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LogicalChannelConfig-DRB ::= SEQUENCE { |  |  |  |  |
| ul-SpecificParameters SEQUENCE { |  |  |  |  |
| priority |  | 6 |  | HI |
|  |  | 13 |  | LO |
| prioritisedBitRate |  | kBps0 | PBR is disabled. |  |
| bucketSizeDuration |  | ms100 |  |  |
| logicalChannelGroup |  | 1 |  | HI |
|  |  | 2 |  | LO |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| HI | Used for DRBs with high logical channel priority |
| LO | Used for DRBs with low logical channel priority |

##### 4.8.2.1.5	MAC configurations

Table 4.8.2.1.5-1: MAC-MainConfig-RBC

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MAC-MainConfig-RBC ::= SEQUENCE { |  |  |  |  |
| ul-SCH-Config |  | Not present |  | SCell_AddMod |
| ul-SCH-Config SEQUENCE { |  |  |  |  |
| maxHARQ-Tx |  | n5 |  |  |
| periodicBSR-Timer |  | sf20 |  |  |
|  |  | Infinity |  | no_periodic_BSR_or_PHR |
| retxBSR-Timer |  | sf320 |  |  |
|  |  | sf10240 |  | no_periodic_BSR_or_PHR |
| ttiBundling |  | FALSE |  |  |
| } |  |  |  |  |
| drx-Config |  | Not present |  | NOT pc_FeatrGrp_5 or SCell_AddMod |
| drx-Config CHOICE { |  |  |  | pc_FeatrGrp_5 AND DRX_S |
| setup SEQUENCE { |  |  |  |  |
| onDurationTimer |  | psf2 |  |  |
|  |  | psf5 |  | For SIG and HalfDuplex |
| drx-InactivityTimer |  | psf100 |  |  |
| drx-RetransmissionTimer |  | psf16 |  |  |
| longDRX-CycleStartOffset CHOICE { |  |  | sf40 typical value in real network for real-time services. |  |
| sf40 |  | 4 |  | For SIG |
|  |  | 0 |  | For RF |
| } |  |  |  |  |
| shortDRX |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| drx-Config CHOICE { |  |  |  | pc_FeatrGrp_5 AND DRX_L |
| setup SEQUENCE { |  |  |  |  |
| onDurationTimer |  | psf6 |  |  |
| drx-InactivityTimer |  | psf1920 |  |  |
| drx-RetransmissionTimer |  | psf16 |  |  |
| longDRX-CycleStartOffset CHOICE { |  |  | sf1280 typical value in real network for best-effort services. |  |
| sf1280 |  | 4 |  | For SIG |
|  |  | 0 |  | For RF |
| } |  |  |  |  |
| shortDRX |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| timeAlignmentTimerDedicated |  | sf750 |  |  |
| shortTTI-AndSPT-r15 |  | Not present |  |  |
| shortTTI-AndSPT-r15 CHOICE { |  |  |  | Short-TTI |
| setup SEQUENCE { |  |  |  |  |
| periodicBSR-Timer-r15 |  | sf20 |  |  |
| proc-Timeline-r15 |  | nplus4set1 |  | UE_ProcessingTimeline_Set1 |
|  |  | nplus6set2 |  | UE_ProcessingTimeline_ Set2 |
| } |  |  |  |  |
| } |  |  |  |  |
| phr-Config CHOICE { |  |  |  | no_periodic_BSR_or_PHR |
| release |  | NULL |  |  |
| } |  |  |  |  |
| phr-Config |  | Not present |  | SCell_AddMod and RF |
| phr-Config CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| periodicPHR-Timer |  | sf500 |  |  |
| prohibitPHR-Timer |  | sf200 |  |  |
| dl-PathlossChange |  | dB3 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sr-ProhibitTimer-r9 |  | Not present |  |  |
| mac-MainConfig-v1020 |  | Not present |  |  |
| mac-MainConfig-v1020 SEQUENCE { |  |  |  | SCell_AddMod |
| sCellDeactivationTimer-r10 |  | rf32 |  |  |
| extendedBSR-Sizes-r10 |  | setup |  |  |
| extendedPHR-r10 |  | setup |  |  |
| } |  |  |  |  |
| stag-ToReleaseList-r11 |  | Not present |  |  |
| stag-ToAddModList-r11 |  | Not present |  |  |
| drx-Config-v1130 |  | Not present |  |  |
| e-HARQ-Pattern-r12 |  | Not present |  |  |
| dualConnectivityPHR |  | Not present |  |  |
| dualConnectivityPHR CHOICE { |  |  |  | SCG_Add_EN-DC |
| setup SEQUENCE { |  |  |  |  |
| phr-ModeOtherCG-r12 |  | real |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| logicalChannelSR-Config-r12 CHOICE {} |  | Not present |  |  |
| extendedPHR2-r13 |  | Not present |  |  |
| eDRX-Config-CycleStartOffset-r13 CHOICE {} |  | Not present |  |  |
| eDRX-Config-CycleStartOffset-r13 CHOICE {} |  | Not present |  |  |
| drx-Config-r13 CHOICE {} |  | Not present |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| DRX_S | Used for DRX configuration with small DRX cycle length |
| DRX_L | Used for DRX configuration with large DRX cycle length |
| no_periodic_BSR_or_PHR | Used to disable BSR and PHR |
| For SIG | Used for DRX configuration in Protocol testing |
| For RF | Used for DRX configuration in RRM testing |
| SCell_AddMod | Addition or modification of SCell |
| HalfDuplex | Used during Type B HalfDuplexOperation |
| SCG_Add_EN-DC | Used during SCG addition of NR cell on EN-DC test cases |
| Short-TTI | When short TTI is used |
| UE_ProcessingTimeline_Set1 | Used for UE capability supporting {subslot, subslot} and processing timeline Set1 |
| UE_ProcessingTimeline_Set2 | Used for UE capability supporting {subslot, subslot} and processing timeline Set2 |

Table 4.8.2.1.5-2: MAC-MainConfig-SRB

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MAC-MainConfig-SRB ::= SEQUENCE { |  |  |  |  |
| ul-SCH-Config SEQUENCE { |  |  |  |  |
| maxHARQ-Tx |  | n5 |  |  |
| periodicBSR-Timer |  | sf20 |  |  |
| retxBSR-Timer |  | sf320 |  |  |
| ttiBundling |  | FALSE |  |  |
| } |  |  |  |  |
| drx-Config CHOICE { |  |  |  |  |
| release |  | NULL |  |  |
| } |  |  |  |  |
| timeAlignmentTimerDedicated |  | sf750 |  |  |
| phr-Config CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| periodicPHR-Timer |  | sf500 |  |  |
| prohibitPHR-Timer |  | sf200 |  |  |
| dl-PathlossChange |  | dB3 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

##### 4.8.2.1.6	Physical Layer configurations

Table 4.8.2.1.6-1: PhysicalConfigDedicated-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  |  |  | Value/remark |  |  | Comment |  | Condition |  |  |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |
| pdsch-ConfigDedicated |  |  |  | PDSCH-ConfigDedicated-DEFAULT |  |  | See subclause 4.6.3 |  | SRB1 or HO-TO-EUTRA or PSCell_Mod or PSCell_Add_Split_DRB or PSCell_Add_SCG_DRB |  |  |
|  |  |  |  | Not present |  |  |  |  | RBC or RBC-HO or SCell_AddMod |  |  |
| pucch-ConfigDedicated |  |  |  | PUCCH-ConfigDedicated-DEFAULT |  |  | See subclause 4.6.3 |  | SRB1 and not (CE Mode A or CE Mode B)) or RBC-HO or HO-TO-EUTRA or PSCell_Mod or PSCell_Add_Split_DRB or PSCell_Add_SCG_DRB |  |  |
|  |  |  |  | Not present |  |  |  |  | RBC or SCell_AddMod or CEmodeA or CEmodeB |  |  |
| pusch-ConfigDedicated |  |  |  | PUSCH-ConfigDedicated-DEFAULT |  |  | See subclause 4.6.3 |  | SRB1 or HO-TO-EUTRA or PSCell_Mod or PSCell_Add_Split_DRB or PSCell_Add_SCG_DRB |  |  |
|  |  |  |  | Not present |  |  |  |  | RBC or RBC-HO or SCell_AddMod |  |  |
| uplinkPowerControlDedicated |  |  |  | UplinkPowerControlDedicated-DEFAULT |  |  | See subclause 4.6.3 |  | SRB1 or HO-TO-EUTRA or PSCell_Mod or PSCell_Add_Split_DRB or PSCell_Add_SCG_DRB |  |  |
|  |  |  |  | Not present |  |  |  |  | RBC or RBC-HO or SCell_AddMod |  |  |
| tpc-PDCCH-ConfigPUCCH |  |  |  | Not present |  |  |  |  | SRB1 or SCell_AddMod |  |  |
|  |  |  |  | TPC-PDCCH-Config-DEFAULT using condition PUCCH |  |  | See subclause 4.6.3 |  | RBC or RBC-HO or HO-TO-EUTRA or PSCell_Mod or PSCell_Add_Split_DRB or PSCell_Add_SCG_DRB |  |  |
| tpc-PDCCH-ConfigPUSCH |  |  |  | Not present |  |  |  |  | SRB1 SCell_AddMod |  |  |
|  |  |  |  | TPC-PDCCH-Config-DEFAULT using condition PUSCH |  |  | See subclause 4.6.3 |  | RBC or RBC-HO or HO-TO-EUTRA or PSCell_Mod or PSCell_Add_Split_DRB or PSCell_Add_SCG_DRB |  |  |
| cqi-ReportConfig |  |  |  | CQI-ReportConfig-DEFAULT |  |  | See subclause 4.6.3 |  | SRB1 or HO-TO-EUTRA or PSCell_Mod or PSCell_Add_Split_DRB or PSCell_Add_SCG_DRB |  |  |
|  |  |  |  | CQI-ReportConfig- DEFAULT using condition CQI_PERIODIC |  |  | See subclause 4.6.3 |  | (RBC or RBC-HO) and not RBC-HO-CA |  |  |
|  |  |  |  | Not present |  |  |  |  | CA HalfDuplex |  |  |
| soundingRS-UL-ConfigDedicated |  |  |  | Not present |  |  |  |  | SRB1 or HO-TO-EUTRA SCell_AddMod PSCell_AddMod HalfDuplex |  |  |
|  |  |  |  | SoundingRS-Ul-ConfigDedicated-DEFAULT |  |  | See subclause 4.6.3 |  | RBC or RBC-HO |  |  |
| antennaInfo CHOICE { |  |  |  |  |  |  |  |  |  |  |  |
| explicitValue SEQUENCE { |  |  |  |  |  |  |  |  | 2TX or 4TX |  |  |
| transmissionMode |  |  |  | tm3 |  |  |  |  |  |  |  |
| codebookSubsetRestriction CHOICE { |  |  |  |  |  |  |  |  |  |  |  |
| n2TxAntenna-tm3 |  |  |  | '11' |  |  | BIT STRING (SIZE (2)) |  | 2TX |  |  |
| n4TxAntenna-tm3 |  |  |  | '1111' |  |  | BIT STRING (SIZE (4)) |  | 4TX |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |  |  |  |  |  |  |  |  |
| Release |  |  |  | NULL |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |
| defaultValue |  |  |  | NULL |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |
| schedulingRequestConfig |  |  |  | SchedulingRequest-Config-DEFAULT |  |  | See subclause 4.6.3 |  | SRB1 or RBC-H or HO-TO-EUTRA |  |  |
|  |  |  |  | Not present |  |  |  |  | RBC SCell_AddMod PSCell_AddMod |  |  |
| cqi-ReportConfig-v920 |  | Not present |  |  |  |  |  |  |  |  |
| antennaInfo-v920 |  | Not present |  |  |  |  |  |  |  |  |
| antennaInfo-r10 |  | Not present |  |  |  |  |  |  |  |  |
| antennaInfoUL-r10 |  | Not present |  |  |  |  |  |  |  |  |
| cif-Presence-r10 |  | FALSE |  |  |  |  |  |  | SCell_AddMod |  |
|  |  | Not present |  |  |  |  |  |  |  |  |
| cqi-ReportConfig-r10 |  | CQI-ReportConfig-r10-DEFAULT |  |  |  |  |  |  | SCell_AddMod or RBC-HO-CA |  |
|  |  | Not present |  |  |  |  |  |  |  |  |
| csi-RS-Config-r10 |  | Not present |  |  |  |  |  |  |  |  |
| pucch-ConfigDedicated-v1020 |  | PUCCH-ConfigDedicated-v1020-DEFAULT |  |  |  |  |  |  | SCell_AddMod |  |
| pusch-ConfigDedicated-v1020 |  | Not present |  |  |  |  |  |  |  |  |
| schedulingRequestConfig-v1020 |  | Not present |  |  |  |  |  |  |  |  |
| soundingRS-UL-ConfigDedicated-v1020 |  | Not present |  |  |  |  |  |  |  |  |
| soundingRS-UL-ConfigDedicatedAperiodic-r10 |  | Not present |  |  |  |  |  |  |  |  |
| uplinkPowerControlDedicated-v1020 |  | UplinkPowerControlDedicated-v1020-DEFAULT |  |  |  |  |  |  | SCell_AddMod and (TxD or SRSAp) |  |
|  |  | Not present |  |  |  |  |  |  |  |  |
| additionalSpectrumEmissionCA-r10 |  | Not present |  |  |  |  |  |  |  |  |
| additionalSpectrumEmissionCA-r10 |  | Not present |  |  |  |  |  |  | SCell_AddMod and Intra-BC CA |  |
| additionalSpectrumEmissionCA-r10 CHOICE { |  |  |  |  |  |  |  |  | SCell_AddMod and Inter-B CA and UL CA |  |
| setup SEQUENCE { |  |  |  |  |  |  |  |  |  |  |
| additionalSpectrumEmissionPCell-r10 |  | Not present |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |
| epdcch-Config-r11 |  | EPDCCH-Config-r11-DEFAULT |  |  |  |  |  |  | ePDCCH or (Not(RBC) and (CE Mode A or CE Mode B)) |  |
|  |  | Not present |  |  |  |  |  |  |  |  |
| csi-RS-ConfigNZPToReleaseList-r11 |  | Not present |  |  |  |  |  |  |  |  |
| csi-RS-ConfigNZPToAddModList-r11 |  | Not present |  |  |  |  |  |  |  |  |
| csi-RS-ConfigNZPToAddModList-r11 SEQUENCE (SIZE (1..maxCSI-RS-NZP-r11)) OF { |  | 1 entry |  |  |  |  |  |  | DL_CoMP |  |
| CSI-RS-ConfigNZP-r11[1] |  | CSI-RS-ConfigNZP-r11-DEFAULT |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |
| csi-RS-ConfigZPToReleaseList-r11 |  | Not present |  |  |  |  |  |  |  |  |
| csi-RS-ConfigZPToAddModList-r11 |  | Not present |  |  |  |  |  |  |  |  |
| csi-RS-ConfigZPToAddModList-r11 SEQUENCE (SIZE (1..maxCSI-RS-ZP-r11)) OF { |  | 1 entry |  |  |  |  |  |  | DL_CoMP |  |
| CSI-RS-ConfigZP-r11[1] |  | CSI-RS-ConfigZP-r11-DEFAULT |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |
| pdsch-ConfigDedicated-v1130 |  | PDSCH-ConfigDedicated-v1130-DEFAULT |  |  |  |  |  |  | DL_CoMP |  |
|  |  | Not present |  |  |  |  |  |  |  |  |
| cqi-ReportConfig-v1130 |  | CQI-ReportConfig-v1130-DEFAULT |  |  |  |  |  |  | DL_CoMP |  |
|  |  | Not present |  |  |  |  |  |  |  |  |
| pusch-EnhancementsConfig-r14 |  |  | PUSCH-EnhancementsConfig-r14-DEFAULT |  |  |  |  |  | eVoLTE |  |  |
|  |  |  | Not present |  |  |  |  |  |  |  |  |
| pucch-ConfigDedicated-v1130 |  | PUCCH-ConfigDedicated-v1130-DEFAULT |  |  |  |  |  |  | UL_CoMP |  |
|  |  | Not present |  |  |  |  |  |  |  |  |
| pusch-ConfigDedicated-v1130 |  | PUSCH-ConfigDedicated-v1130-DEFAULT |  |  |  |  |  |  | UL_CoMP |  |
|  |  | Not present |  |  |  |  |  |  |  |  |
| uplinkPowerControlDedicated-v1130 |  | UplinkPowerControlDedicated-v1130-DEFAULT |  |  |  |  |  |  | UL_CoMP |  |
|  |  | Not present |  |  |  |  |  |  |  |  |
| cqi-ReportConfigPCell-v1250 |  | CQI-ReportConfig-v1250-DEFAULT |  |  |  |  |  |  | DL 256QAM |  |
|  |  | Not present |  |  |  |  |  |  |  |  |
| uplinkPowerControlDedicated-v1250 |  | Not present |  |  |  |  |  |  |  |  |
| pusch-ConfigDedicated-v1250 |  | Not present |  |  |  |  |  |  |  |  |
| csi-RS-Config-v1250 |  | Not present |  |  |  |  |  |  |  |  |
| pdsch-ConfigDedicated-v1280 |  | Not present |  |  |  |  |  |  |  |  |
| pdsch-ConfigDedicated-v1310 |  | Not present |  |  |  |  |  |  |  |  |
| pucch-ConfigDedicated-r13 |  | Not present |  |  |  |  |  |  |  |  |
|  |  | PUCCH-ConfigDedicated- r13-DEFAULT |  |  |  |  |  |  | Not (RBC) and ( CEmodeA or CemodeB) |  |
| pusch-ConfigDedicated-r13 |  | Not present |  |  |  |  |  |  |  |  |
| pdcch-CandidateReductions-r13 |  | Not present |  |  |  |  |  |  |  |  |
| cqi-ReportConfig-v1310 |  | Not present |  |  |  |  |  |  |  |  |
| soundingRS-UL-ConfigDedicated-v1310 |  | Not present |  |  |  |  |  |  |  |  |
| soundingRS-UL-ConfigDedicatedUpPTsExt-r13 |  | Not present |  |  |  |  |  |  |  |  |
| soundingRS-UL-ConfigDedicatedAperiodic-v1310 |  | Not present |  |  |  |  |  |  |  |  |
| soundingRS-UL-ConfigDedicatedAperiodicUpPTsExt-r13 |  | Not present |  |  |  |  |  |  |  |  |
| csi-RS-Config-v1310 |  | Not present |  |  |  |  |  |  |  |  |
| ce-Mode-r13 CHOICE {} |  | Not present |  |  |  |  |  |  |  |  |
| ce-Mode-r13 CHOICE { |  |  |  |  |  |  |  |  | Not (RBC) and ( CEmodeA OR CemodeB) |  |
| Setup |  | ce-ModeA |  |  |  |  |  |  | CEmodeA |  |
| setup |  | ce-ModeB |  |  |  |  |  |  | CEmodeB |  |
| } |  |  |  |  |  |  |  |  |  |  |
| physicalConfigDedicatedSTTI-r15 |  | Not present |  |  |  |  |  |  |  |  |
| physicalConfigDedicatedSTTI-r15 CHOICE { |  |  |  |  |  |  |  |  | Short-TTI |  |
| setup SEQUENCE{ |  |  |  |  |  |  |  |  |  |  |
| antennaInfoDedicatedSTTI-r15 CHOICE { |  |  |  |  |  |  |  |  |  |  |
| setup SEQUENCE{ |  |  |  |  |  |  |  |  |  |  |
| transmissionModeDL-MBSFN-r15 |  | tm3 |  |  |  |  |  |  |  |  |
| transmissionModeDL-nonMBSFN-r15 |  | tm3 |  |  |  |  |  |  |  |  |
| maxLayersMIMO-STTI-r15 |  | fourLayers |  |  |  |  |  |  |  |  |
| slotSubslotPDSCH-TxDiv-2Layer-r15 |  | Not present |  |  |  |  |  |  |  |  |
| slotSubslotPDSCH-TxDiv-4Layer-r15 |  | Not present |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |
| antennaInfoUL-STTI-r15 SEQUENCE { |  |  |  |  |  |  |  |  |  |  |
| transmissionModeUL-STTI-r15 |  | tm1 |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |
| pucch-ConfigDedicated-v1530 |  | Not present |  |  |  |  |  |  |  |  |
| schedulingRequestConfig-v1530 |  | Not present |  |  |  |  |  |  |  |  |
| uplinkPowerControlDedicatedSTTI-r15 SEQUENCE{ |  |  |  |  |  |  |  |  |  |  |
| accumulationEnabledSTTI-r15 |  | TRUE |  |  |  |  |  |  |  |  |
| uplinkPower-CSIPayload |  | FALSE |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |
| semiOpenLoopSTTI-r15 |  | Not present |  |  |  |  |  |  |  |  |
| slotOrSubslotPDSCH-Config-r15 |  | SlotOrSubslotPDSCH-Config-r15-DEFAULT |  |  |  |  |  |  |  |  |
| slotOrSubslotPUSCH-Config-r15 |  | SlotOrSubslotPUSCH-Config-r15-DEFAULT |  |  |  |  |  |  |  |  |
| spdcch-Config-r15 |  | SPDCCH-Config-r15-DEFAULT |  |  |  |  |  |  |  |  |
| spucch-Config-r15 |  | SPUCCH-Config-r15-DEFAULT |  |  |  |  |  |  |  |  |
| srs-DCI7-TriggeringConfig-r15 |  | Not present |  |  |  |  |  |  |  |  |
| shortProcessingTime-r15 |  | Not present |  |  |  |  |  |  |  |  |
| shortTTI-r15 SEQUENCE{ |  |  |  |  |  |  |  |  |  |  |
| dl-STTI-Length-r15 |  | slot |  |  |  |  |  |  |  |  |
| ul-STTI-Length-r15 |  | slot |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |
| uplinkPowerControlDedicated-v1530 |  | Not present |  |  |  |  |  |  |  |  |
| uplinkPowerControlDedicated-v1530 |  | UplinkPowerControlDedicated-v1530-DEFAULT |  |  |  |  |  |  | Aerial UE |  |
| } |  |  |  |  |  |  |  |  |  |  |
| Note: For signalling test, table 6.6B.1.1.1-1 is applied. |  |  |  |  |  |  |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| SRB1 | Used at configuration of SRB1 during RRC connection (re-)establishment |
| RBC | Used at configuration of a radio bearer combination during SRB2+DRB establishment |
| 2TX | Used for cells with two antenna ports |
| 4TX | Used for cells with four antenna ports |
| RBC-HO | Used during Handover |
| RBC-HO-CA | Used during Handover when Carrier Aggregation is configured for the source cells |
| SCell_AddMod | Addition or modification of SCell |
| HO-TO-EUTRA | Inter-RAT handover to E-UTRA |
| ePDCCH | Used at configuration of ePDCCH |
| DL_CoMP | Used for DL CoMP |
| UL_CoMP | Used for UL CoMP |
| CA | For Carrier Aggregation Test cases |
| TxD | When PUCCH Transmission Diversity is performed. |
| SRSAp | Aperiodic SRS configured |
| Intra-BC CA | Intra-band contiguous Carrier Aggregation |
| Inter-B CA | Inter-band Carrier Aggregation |
| UL CA | When UL Carrier Aggregation is used. |
| PSCell_Mod | Modification of PSCell |
| PSCell_Add_Split_DRB | Add PSCell(s) and setup of Split DRB |
| PSCell_Add_SCG_DRB | Add PSCell(s) and setup of SCG DRB |
| CEmodeA | Used for CE mode A testing |
| CEmodeB | Used for CE mode B testing |
| HalfDuplex | Used during Type B HalfDuplexOperation |
| DL 256QAM | When DL 256QAM is used |
| eVoLTE | When VoLTE enhancement is used. |
| Aerial UE | For Aerial vehicles |
| Short-TTI | When short TTI is used |

Table 4.8.2.1.6-1A: PhysicalConfigDedicated-eIMTA

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  |  | Value/remark |  | Comment | Condition |  |
| PhysicalConfigDedicated-eIMTA ::= SEQUENCE { |  |  |  |  |  |  |  |
| pdsch-ConfigDedicated |  |  | PDSCH-ConfigDedicated-DEFAULT |  | See subclause 4.6.3 | SRB1 or HO-TO-EUTRA |  |
|  |  |  | Not present |  |  | RBC or RBC-HO SCell_AddMod |  |
| pucch-ConfigDedicated |  |  | PUCCH-ConfigDedicated-DEFAULT |  | See subclause 4.6.3 | SRB1 or RBC-HO or HO-TO-EUTRA |  |
|  |  |  | Not present |  |  | RBC SCell_AddMod |  |
| pusch-ConfigDedicated |  |  | PUSCH-ConfigDedicated-DEFAULT |  | See subclause 4.6.3 | SRB1 or HO-TO-EUTRA |  |
|  |  |  | Not present |  |  | RBC or RBC-HO SCell_AddMod |  |
| uplinkPowerControlDedicated |  |  | UplinkPowerControlDedicated-DEFAULT |  | See subclause 4.6.3 | SRB1 or HO-TO-EUTRA |  |
|  |  |  | Not present |  |  | RBC or RBC-HO SCell_AddMod |  |
| tpc-PDCCH-ConfigPUCCH |  |  | Not present |  |  | SRB1 SCell_AddMod |  |
|  |  |  | TPC-PDCCH-Config-DEFAULT using condition PUCCH |  | See subclause 4.6.3 | RBC or RBC-HO or HO-TO-EUTRA |  |
| tpc-PDCCH-ConfigPUSCH |  |  | Not present |  |  | SRB1 SCell_AddMod |  |
|  |  |  | TPC-PDCCH-Config-DEFAULT using condition PUSCH |  | See subclause 4.6.3 | RBC or RBC-HO or HO-TO-EUTRA |  |
| cqi-ReportConfig |  |  | CQI-ReportConfig-DEFAULT |  | See subclause 4.6.3 | SRB1 or HO-TO-EUTRA |  |
|  |  |  | CQI-ReportConfig- DEFAULT using condition CQI_PERIODIC |  | See subclause 4.6.3 | RBC or RBC-HO |  |
|  |  |  | Not present |  |  | CA |  |
| soundingRS-UL-ConfigDedicated |  |  | Not present |  |  | SRB1 or HO-TO-EUTRA SCell_AddMod |  |
|  |  |  | SoundingRS-Ul-ConfigDedicated-DEFAULT |  | See subclause 4.6.3 | RBC or RBC-HO |  |
| antennaInfo CHOICE { |  |  |  |  |  |  |  |
| explicitValue SEQUENCE { |  |  |  |  |  | 2TX |  |
| transmissionMode |  |  | tm3 |  |  |  |  |
| codebookSubsetRestriction CHOICE { |  |  |  |  |  |  |  |
| n2TxAntenna-tm3 |  |  | '11' |  | BIT STRING (SIZE (2)) |  |  |
| } |  |  |  |  |  |  |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |  |  |  |  |
| Release |  |  | NULL |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| defaultValue |  |  | NULL |  |  |  |  |
| } |  |  |  |  |  |  |  |
| schedulingRequestConfig |  |  | SchedulingRequest-Config-DEFAULT |  | See subclause 4.6.3 | SRB1 or RBC-H or HO-TO-EUTRA |  |
|  |  |  | Not present |  |  | RBC SCell_AddMod |  |
| cqi-ReportConfig-v920 | Not present |  |  |  |  |  |
| antennaInfo-v920 | Not present |  |  |  |  |  |
| antennaInfo-r10 | Not present |  |  |  |  |  |
| antennaInfoUL-r10 | Not present |  |  |  |  |  |
| cif-Presence-r10 | FALSE |  |  |  |  | SCell_AddMod |
|  | Not present |  |  |  |  |  |
| cqi-ReportConfig-r10 | CQI-ReportConfig-r10-DEFAULT |  |  |  |  | SCell_AddMod |
|  | Not present |  |  |  |  |  |
| csi-RS-Config-r10 | Not present |  |  |  |  |  |
| pucch-ConfigDedicated-v1020 | PUCCH-ConfigDedicated-v1020-DEFAULT |  |  |  |  | SCell_AddMod |
| pusch-ConfigDedicated-v1020 | Not present |  |  |  |  |  |
| schedulingRequestConfig-v1020 | Not present |  |  |  |  |  |
| soundingRS-UL-ConfigDedicated-v1020 | Not present |  |  |  |  |  |
| soundingRS-UL-ConfigDedicatedAperiodic-r10 | Not present |  |  |  |  |  |
| uplinkPowerControlDedicated-v1020 | UplinkPowerControlDedicated-v1020-DEFAULT |  |  |  |  | SCell_AddMod and (TxD or SRSAp) |
|  | Not present |  |  |  |  |  |
| additionalSpectrumEmissionCA-r10 | Not present |  |  |  |  |  |
| additionalSpectrumEmissionCA-r10 | Not present |  |  |  |  | SCell_AddMod and Intra-BC CA |
| additionalSpectrumEmissionCA-r10 CHOICE { |  |  |  |  |  | SCell_AddMod and Inter-B CA and UL CA |
| setup SEQUENCE { |  |  |  |  |  |  |
| additionalSpectrumEmissionPCell-r10 | 1 (CA_NS_01) |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| epdcch-Config-r11 | EPDCCH-Config-r11- eIMTA |  |  |  |  | ePDCCH |
|  | Not present |  |  |  |  |  |
| csi-RS-ConfigNZPToReleaseList-r11 | Not present |  |  |  |  |  |
| csi-RS-ConfigNZPToAddModList-r11 | Not present |  |  |  |  |  |
| csi-RS-ConfigNZPToAddModList-r11 SEQUENCE (SIZE (1..maxCSI-RS-NZP-r11)) OF { | 1 entry |  |  |  |  | DL_CoMP |
| CSI-RS-ConfigNZP-r11[1] | CSI-RS-ConfigNZP-r11-DEFAULT |  |  |  |  |  |
| } |  |  |  |  |  |  |
| csi-RS-ConfigZPToReleaseList-r11 | Not present |  |  |  |  |  |
| csi-RS-ConfigZPToAddModList-r11 | Not present |  |  |  |  |  |
| csi-RS-ConfigZPToAddModList-r11 SEQUENCE (SIZE (1..maxCSI-RS-ZP-r11)) OF { | 1 entry |  |  |  |  | DL_CoMP |
| CSI-RS-ConfigZP-r11[1] | CSI-RS-ConfigZP-r11-DEFAULT |  |  |  |  |  |
| } |  |  |  |  |  |  |
| pdsch-ConfigDedicated-v1130 | PDSCH-ConfigDedicated-v1130-DEFAULT |  |  |  |  | DL_CoMP |
|  | Not present |  |  |  |  |  |
| cqi-ReportConfig-v1130 | CQI-ReportConfig-v1130-eIMTA |  |  |  |  | DL_CoMP |
|  | Not present |  |  |  |  |  |
| pucch-ConfigDedicated-v1130 | PUCCH-ConfigDedicated-v1130-DEFAULT |  |  |  |  | UL_CoMP |
|  | Not present |  |  |  |  |  |
| pusch-ConfigDedicated-v1130 | PUSCH-ConfigDedicated-v1130-DEFAULT |  |  |  |  | UL_CoMP |
|  | Not present |  |  |  |  |  |
| uplinkPowerControlDedicated-v1130 | UplinkPowerControlDedicated-v1130-DEFAULT |  |  |  |  | UL_CoMP |
|  | Not present |  |  |  |  |  |
| antennaInfo-v1250 | Not present |  |  |  |  |  |
| eimta-MainConfig-r12 | EIMTA-MainConfig-r12-DEFAULT |  |  |  |  | eIMTA |
|  | Not present |  |  |  |  |  |
| eimta-MainConfigPCell-r12 | EIMTA-MainConfigServCell-r12-DEFAULT |  |  |  |  | eIMTA |
|  | Not present |  |  |  |  |  |
| pucch-ConfigDedicated-v1250 | PUCCH-ConfigDedicated-v1250-DEFAULT |  |  |  |  | eIMTA |
|  | Not present |  |  |  |  |  |
| cqi-ReportConfigPCell-v1250 | CQI-ReportConfig-v1250 -DEFAULT |  |  |  |  | eIMTA |
|  | Not present |  |  |  |  |  |
| uplinkPowerControlDedicated-v1250 | UplinkPowerControlDedicated-v1250-DEFAULT |  |  |  |  | eIMTA |
|  | Not present |  |  |  |  |  |
| pusch-ConfigDedicated-v1250 | PUSCH-ConfigDedicated-v1250-DEFAULT |  |  |  |  | eIMTA |
|  | Not present |  |  |  |  |  |
| csi-RS-Config-v1250 | Not present |  |  |  |  |  |
| } |  |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| SRB1 | Used at configuration of SRB1 during RRC connection (re-)establishment |
| RBC | Used at configuration of a radio bearer combination during SRB2+DRB establishment |
| 2TX | Used for cells with two antenna ports |
| RBC-HO | Used during Handover |
| RBC-HO-CA | Used during Handover when Carrier Aggregation is configured for the source cells |
| SCell_AddMod | Addition or modification of SCell |
| HO-TO-EUTRA | Inter-RAT handover to E-UTRA |
| ePDCCH | Used at configuration of ePDCCH |
| DL_CoMP | Used for DL CoMP |
| UL_CoMP | Used for UL CoMP |
| CA | For Carrier Aggregation Test cases |
| TxD | When PUCCH Transmission Diversity is performed. |
| SRSAp | Aperiodic SRS configured |
| Intra-BC CA | Intra-band contiguous Carrier Aggregation |
| Inter-B CA | Inter-band Carrier Aggregation |
| UL CA | When UL Carrier Aggregation is used. |
| eIMTA | eIMTA cell environment |

##### 4.8.2.1.7	DRB configurations

Table 4.8.2.1.7-1: DRB-ToAddMod-DEFAULT(bid)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DRB-ToAddMod-DEFAULT(bid) ::= SEQUENCE { |  |  | bid is the bearer identity (1..8) |  |
| eps-BearerIdentity |  | bid+4 |  |  |
| drb-Identity |  | bid |  |  |
| pdcp-Config |  | PDCP-Config-DRB-AM |  | AM |
|  |  | PDCP-Config-DRB-UM |  | UM |
| rlc-Config |  | RLC-Config-DRB-AM |  | AM |
|  |  | RLC-Config-DRB-UM |  | UM |
| logicalChannelIdentity |  | bid+2 |  |  |
| logicalChannelConfig |  | LogicalChannelConfig-DRB using condition LO |  | UM |
|  |  | LogicalChannelConfig-DRB using condition HI |  | AM |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| AM | Used for AM DRB |
| UM | Used for UM DRB |

Table 4.8.2.1.7-2: DRB-ToAddMod-EN-DC-Split

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DRB-ToAddMod-EN-DC-Split ::= SEQUENCE { |  |  |  |  |
| eps-BearerIdentity |  | 6 |  |  |
| drb-Identity |  | 2 |  |  |
| pdcp-Config |  | Not present |  |  |
| rlc-Config |  | RLC-Config-DRB-AM |  |  |
| logicalChannelIdentity |  | Not present |  |  |
| logicalChannelConfig |  | LogicalChannelConfig-DRB using condition HI |  |  |
| } |  |  |  |  |

#### 4.8.2.2	SRB and DRB combinations

##### 4.8.2.2.1	Combinations on DL-SCH and UL-SCH

4.8.2.2.1.1	SRB1 and SRB2 for DCCH + n x AM DRB + m x UM DRB, where n=1..N and m=0..M

This SRB and DRB combination is setup with UE Registration procedure and the Generic Radio Bearer Establishment or Generic Radio Bearer Establishment, UE Test Mode Activated procedure using specific message content - the default RRCConnectionReconfiguration message with condition SRB2-DRB(n, m).

### 4.8.3	UTRA reference radio parameters and combinations

Table 4.8.3-1 defines UTRA reference radio parameters and combinations to be used in E-UTRA and UTRA inter-RAT test cases.

Table 4.8.3-1: UTRA reference radio parameters and combinations

| Condition | Reference configuration | Comment |
| --- | --- | --- |
| UTRA Speech | TS 34.108 subclause 6.10.2.4.1.4 | Conversational / speech / UL:12.2 DL:12.2 kbps / CS RAB + UL:3.4 DL:3.4 kbps SRBs for DCCH |
| UTRA FDD PS RB | TS 34.108 subclause 6.10.2.4.1.26 | Interactive or background / UL:64 DL: 64 kbps / PS RAB + UL:3.4 DL:3.4 kbps SRBs for DCCH |
| UTRA HSDPA RB | TS 34.108 subclause 6.10.2.4.5.1 | Interactive or background / UL:64 DL: [max bit rate depending on UE category] / PS RAB + UL:3.4 DL:3.4 kbps SRBs for DCCH |
| UTRA HSUPA/HSDPA RB | TS 34.108 subclause 6.10.2.4.6.3 | Streaming or interactive or background / UL: [max bit rate depending on UE category and TTI] DL: [max bit rate depending on UE category] / PS RAB + UL: [max bit rate depending on UE category and TTI] DL: [max bit rate depending on UE category] SRBs for DCCH on E-DCH and HS-DSCH |
| UTRA PS RB + Speech | TS 34.108 subclause 6.10.2.4.1.40 | Conversational / speech / UL:12.2 DL:12.2 kbps / CS RAB + Interactive or background / UL:64 DL:64 kbps / PS RAB+ UL:3.4 DL: 3.4 kbps SRBs for DCCH |

### 4.8.4	GERAN reference PDP context parameters

Table 4.8.4-1 defines GERAN reference PDP context parameters to be used in E-UTRA and GERAN inter-RAT test cases.

Table 4.8.4-1: GERAN reference PDP context parameters

| Condition | Reference configuration | Comment |
| --- | --- | --- |
| GPRS | TS 51.010-1 subclause 40.5, Test PDP context3. | Test PDP context3 is the default Test PDP context which is used in the GERAN Inter-RAT GPRS test cases where no particular Test PDP contexts are specified. Compression is always turned off if nothing else is stated explicitly in the test case. |

## 4.9	Common test USIM, CSIM and ISIM parameters

This clause defines default parameters for programming the elementary files of the test UICC when running conformance test cases defined in 3GPP TS 36.523-1[18].

### 4.9.1	General

See clause 8.1 in 3GPP TS 34.108 [5].

#### 4.9.1.1	Definitions

See clause 8.1.1 in 3GPP TS 34.108 [5].

#### 4.9.1.2	Definition of the test algorithm for authentication

Same as clause 8.1.2 in 3GPP TS 34.108[5].

##### 4.9.1.2.1	Authentication and key derivation in the test USIM, CSIM and ISIM and SS

UE and SS calculate Ck, Ik, AUTN, RES[XRES] as in clause 8.1.2.1 in 3GPP TS 34.108 [5]. Derivation of KASME and other E-UTRA Keys shall be as defined in Annex A of 3GPP TS 33.401 [31], using Key derivation function HMAC-SHA-256 algorithm.

##### 4.9.1.2.2	Generation of re-synchronization parameters in the USIM, CSIM and ISIM

Same as clause 8.1.2.1 in 3GPP TS 34.108[5].

##### 4.9.1.2.3	Using the authentication test algorithm for UE conformance testing

See clause 8.1.2.3 in 3GPP TS 34.108 [5].

### 4.9.2	Default parameters for the test USIM, CSIM and ISIM

Same as clause 8.2 in 3GPP TS 34.108 [5].

### 4.9.3	Default settings for the Elementary Files (EFs)

The format and coding of elementary files of the USIM are defined in 3GPP TS 31.101 [32] and 3GPP TS 31.102 [33] . Those of the ISIM are defined in 3GPP TS 31.101 [32] and 3GPP TS 31.103 [45]. Those of the CSIM are defined in 3GPP2 C.S0065-B [48].

The settings of the elementary files at the MF and USIM ADF (Application DF) level resp. the ISIM ADF (Application DF) level are the same as section 8.3 in 3GPP TS 34.108 [5] resp. section 4.2 in 3GPP TS 34.229-1 Annex E [46] and 3GPP TS 34.229-3 Annex B.1 [47] with the exceptions listed below. The settings of the elementary files at the CSIM ADF (Application DF) level as specified below. Note that some files may be updated by the UE based on information received from the SS.

#### 4.9.3.1	Modified contents of the USIM Elementary Files and additional USIM Elements files at the DF ProSe level

EFAD (Administrative Data)

-	Additional information:

Byte 3 (second byte of additional information):

b3 is used to indicate whether the USIM enables the Public Safety UE to use the ME provisioning parameters for Public Safety usage, in the cases described in TS 24.334 [57].

- b3=1: the ME is authorized to use the parameters stored in the USIM or in the ME for ProSe services for Public Safety usage, as described in TS 24.334 [57] without contacting the ProSe Function.

EFEPSLOCI (EPS location information)

File size:	18 Bytes

Default values:	Bytes 1 to 12 (HEX):	FF FF FF FF FF FF FF FF FF FF FF FF(GUTI)

Bytes 13 to 17 (HEX):	42 F6 18 FF FE (Last visited registered TAI)

Byte 18 (BIN):	00000001 (EPS update status = "not updated")

Bytes 13 to 17: TAI-MCC = 246 (bytes 13 to 14) and TAI-MNC = 81 (byte 15) are frequently used. The TAC (bytes 16 to 17) is set to "FF FE" since this, in conjunction with byte 18 setting of "01", is used to ensure that the UE performs Attach at the beginning of a test.

Bytes in this file (e.g. GUTI in bytes 1 to 12) may be updated as a result of a tracking area update attempt by the UE.

EFEPSNSC (EPS NAS Security Context)

The programming of this EF follows default parameter written in 3GPP TS 31.102 [23], annex E.

EFNASCONFIG (Non Access Stratum Configuration)

File size:	28 Bytes

Default values:	Bytes 1 to 28 (HEX):	FF FF … FF

The programming of this EF follows the specific USIM requirements given in 3GPP TS 36.523-1 [18].

EFUST (USIM Service Table):

| Services |  | Activated | Version |
| --- | --- | --- | --- |
| Service n°101 | ProSe | Optional |  |
| Note: Only ProSe related services indicated. |  |  |  |

EFPROSE_ANN (ProSe Announcing Parameters)

If service n°1 is "available" in the ProSe Service Table (EFPST), this file shall be present.

Two PLMNs are authorised for restricted ProSe direct discovery for public safety. Consequently two records associated with each different PLMN are provided.

PLMN1

File size:	7 Bytes

Default values:	Bytes 1 to 7 (HEX):	A0 05 80 03 00 F1 10

The PLMN code of the operator in which the UE is authorized to use restricted ProSe direct discovery announcing for public safety consists of MCC = 001 and MNC = 01. An authorized announcing range is not specified.

PLMN2

File size:	7 Bytes

Default values:	Bytes 1 to 7 (HEX):	A0 07 80 03 00 F1 10

The PLMN code of the operator in which the UE is authorized to use restricted ProSe direct discovery announcing for public safety consists of MCC = 001 and MNC = 02. An authorized announcing range is not specified.

Model Tag	'82'

Coding: Value = '03' (the UE is authorised for Restricted Model A announcing policy and for Restricted Model B discoverer policy)

EFPROSE_MON (ProSe Monitoring Parameters)

If service n°1 is "available" in the ProSe Service Table (EFPST), this file shall be present.

Two PLMNs are authorised for restricted ProSe direct discovery monitoring for public safety. Consequently two records associated with each different PLMN are provided.

PLMN1

File size:	7 Bytes

Default values:	Bytes 1 to 7 (HEX):	A0 05 80 03 00 F1 10

The PLMN code of the operator in which the UE is authorized to use restricted ProSe direct discovery monitoring for public safety consists of MCC = 001 and MNC = 01.

PLMN2

File size:	7 Bytes

Default values:	Bytes 1 to 7 (HEX):	A0 07 80 03 00 F1 10

The PLMN code of the operator in which the UE is authorized to use ProSe direct discovery monitoring for public safety consists of MCC = 001 and MNC = 02.

Model Tag	'82'

Coding: Value = '03' (the UE is authorised for Restricted Model A monitoring policy and for Restricted Model B discoveree policy)

EFPROSEFUNC (HPLMN ProSe Function)

If service n°2 is "available" in the ProSe Service Table (EFPST), this file shall be present.

File size:	7 Bytes

Default values:	Bytes 1 to 7 (HEX):	80 05 01 AC 16 02 67

The type of the HPLMN ProSe Function address is set to IPv4 and the address equals the IP address of the Internet PDN (= 172.22.2.103).

EFPROSE_RADIO_COM (ProSe Direct Communication Radio Parameters)

If service n°3 is "available" in the ProSe Service Table (EFPST), this file shall be present.

This EF contains the radio parameters to be used for ProSe direct communication for public safety when the UE is not served by E-UTRAN, as described in TS 24.334 [57].

File size:	100 Bytes

Default values:	Bytes 1 to 12 (HEX):	A0 FF 80 12 32 D9 4A 63 55 A3 32

Bytes 13 to 24 (HEX):	D2 D1 63 55 A3 32 D6 0D 63 59 47

Bytes 25 to 36 (HEX):	81 25 00 13 1D DE 02 00 00 00 04 29

Bytes 37 to 48 (HEX):	50 00 00 03 00 18 00 28 C0 00 00 00

Bytes 48 to 60 (HEX):	03 F0 60 03 00 05 00 FF FF 00 00 00

Bytes 61 to 72 (HEX):	1F 84 80 12 B2 D9 4A 63 55 A3 B2 D2

Bytes 73 to 84 (HEX):	D1 63 55 A3 B2 D6 0D 63 59 47 81 25

Bytes 85 to 96 (HEX):	00 13 1D DE 02 00 00 00 04 29 50 00

Bytes 97 to 108 (HEX):	00 03 00 18 00 28 30 00 00 00 03 F0

Bytes 109 to 120 (HEX): 60 03 00 05 00 00 00 FF FF 00 00 1F

Bytes 121 to 122 (HEX):	84

High level requirements

ProSe Direct Communication not served by E-UTRAN:

-	Coding: Value = '03'; indicates that UE is authorised to perform one-to-many and one-to-one ProSe direct communication when not served by E-UTRAN

ProSe Radio parameters information:

First data object:

Geographical Area - Polygon Tag '80'

Point 1

Degrees of latitude:	35.753056

Degrees of longitude:	139.689167

Point 2

Degrees of latitude:	35.735278

Degrees of longitude:	139.689167

Point 3

Degrees of latitude:	35.744167

Degrees of longitude:	139.709167

Radio parameters Tag '81' (First data object)

Coded as SL-Preconfiguration in Table 6.8.1.1-1 condition SLCONF1 using Band 3 and 5MHz bandwidth as example.

Second data object:

Geographical Area - Polygon Tag '80'

Point 1

Degrees of latitude:	-35.753056

Degrees of longitude:	139.689167

Point 2

Degrees of latitude:	-35.735278

Degrees of longitude:	139.689167

Point 3

Degrees of latitude:	-35.744167

Degrees of longitude:	139.669167

Radio parameters Tag '81' (First data object)

Coded as SL-Preconfiguration in Table 6.8.1.1-1 condition SLCONF2 using Band 3 and 5MHz bandwidth as example.

EFPROSE_RADIO_MON (ProSe Direct Discovery Monitoring Radio Parameters)

If service n°4 is "available" in the ProSe Service Table (EFPST), this file shall be present.

This EF contains the radio paramenters to be used for ProSe direct communication for public safety when the UE is not served by E-UTRAN, as described in TS 24.334 [57].

High level requirements

ProSe Direct Discovery monitoring not served by E-UTRAN:

-	Coding: Value = '03'; indicates that UE is authorised to perform ProSe direct discovery model A monitoring and ProSe direct discovery model B discoveree operation when not served by E-UTRAN

ProSe Radio parameters information same as the one in EFPROSE_RADIO_COM

EFPROSE_RADIO_ANN (ProSe Direct Discovery Announcing Radio Parameters

If service n°5 is "available" in the ProSe Service Table (EFPST), this file shall be present.

This EF contains the radio paramenters to be used for ProSe direct communication for public safety when the UE is not served by E-UTRAN, as described in TS 24.334 [57].

High level requirements

ProSe Direct Discovery monitoring not served by E-UTRAN:

-	Coding: Value = '03'; indicates that UE is authorised to perform ProSe direct discovery model A announcing and ProSe direct discovery model B discoverer operation when not served by E-UTRAN

ProSe Radio parameters information same as the one in EFPROSE_RADIO_COMEFPROSE_POLICY (ProSe Policy Parameters)

If service n°6 is "available" in the ProSe Service Table (EFPST), this file shall be present.

This EF contains the policy paramenters to be used for ProSe direct communication for public safety, as described in TS 24.334 [57].

File size:	63 Bytes

Default values:	Bytes 1 to 12 (HEX):	A0 3D 80 03 00 00 01 81 03 00 00 01

Bytes 13 to 24 (HEX):	82 04 E0 00 00 02 83 01 01 84 04 AC

Bytes 25 to 36 (HEX):	16 02 68 85 22 00 00 00 00 00 00 00

Bytes 37 to 48 (HEX):	 00 00 00 00 00 00 00 00 00 00 00 00

Bytes 49 to 60 (HEX):	 00 00 00 00 00 00 00 00 00 00 00 00

Bytes 61 to 63 (HEX):	02 01 10

The value of the ProSe Layer-2 Group ID is 1.

The value of ProSe UE ID is 2.

The value of ProSe Group IP multicast address is 224.0.0.1

Address type value is '01' (indicates IPv4)

The IPv4 address of the source, i.e. Device Under Test is 172.22.2.104.

For the Group related security. The value of PGK is 2, the value of PGK Id is 1, and, the algorithm is 128-EEA1.

EFPROSE_PLMN (ProSe PLMN Parameters)

If service n°3 is "available" in the ProSe Service Table (EFPST), this file shall be present.

Specifies the PLMNs in which the UE is authorised to perform ProSe direct communication when served by E-UTRAN.

Two PLMNs are authorised for ProSe direct communication. Consequently two records associated with each different PLMN are provided.

PLMN1

File size:	7 Bytes

Default values:	Bytes 1 to 7 (HEX):	A0 05 80 03 00 F1 10

The PLMN code of the operator in which the UE is authorized to use ProSe direct communication consists of MCC = 001 and MNC = 01.

PLMN2

File size:	7 Bytes

Default values:	Bytes 1 to 7 (HEX):	A0 07 80 03 00 F1 10

The PLMN code of the operator in which the UE is authorized to use ProSe direct communication consists of MCC = 001 and MNC = 02.

EFPROSE_GC (ProSe Group Counter)

If service n°7 is "available" in the ProSe Service Table (EFPST), this file shall be present.

File size:	10 Bytes

Default values:	Bytes 1 to 9 (HEX):	80 07 00 00 01 00 02 00 03

The value of the ProSe Layer-2 Group ID is 1.

The value of PTK ID is 2.

The Counter for the PGK used in the group is 3.

EFPST (ProSe Service Table)

If service n°101 is "available" in the ProSe Service Table (EFUST), this file shall be present.

File size:	2 Bytes

Default values:	Bytes 1 to 2 (HEX):	24 00

This translates to:

| Services |  | Activated | Version |
| --- | --- | --- | --- |
| Service n°1: | ProSe direct discovery parameters | Optional |  |
| Service n°2: | HPLMN ProSe Function | Optional |  |
| Service n°3: | ProSe Direct Communication radio parameters | Optional |  |
| Service n°4: | ProSe Direct Discovery monitoring radio parameters | Optional |  |
| Service n°5: | ProSe Direct Discovery announcing radio parameters | Optional |  |
| Service n°6: | ProSe policy parameters | Optional |  |
| Service n°7: | ProSe group counter | Optional |  |
| Service n°8: | ProSe Usage Information Reporting configuration | Optional (Note 1) |  |
| Service n°9: | UICC ProSe Direct Communication usage information reporting | Optional |  |
| Service n°10: | ProSe Group Member Discovery parameters | Optional |  |
| Service n°11: | ProSe Relay parameters | Optional (Note 2) |  |
| Note 1: If service n°9 is "available", then service n°8 shall also be "available". Note 2: If service n°10 is "available", then service n°1 shall also be "available". |  |  |  |

EFPROSE_UIRC (ProSe UsageInformationReportingConfiguration)

If service n°8 is "available" in the ProSe Service Table (EFPST), this file shall be present.

This EF contains the description of the configuration to be used by the UE for reporting the usage information for direct communication for public safety, as described in TS 24.334 [57].

ProSe UsageInformationReportingConfiguration parameters information:

| Description | Value | Comments |
| --- | --- | --- |
| ProSe ServerAddress |  | No server address is provided, the UE shall upload the usage information reports to the IP address of the HPLMN ProSe Function. |
| ProSe CollectionPeriod | 1 | The time interval, in unit of minutes, at which the UE shall generate the usage information reports |
| ProSe ReportingWindow | 5 | The time window, in unit of minutes, during which the UE shall upload the usage information report to the server |
| ProSe ReportGroupParameters | 1 | The UE shall report the group parameters in the usage information |
| ProSe ReportTimeStampsFirstTransmissionAndReception | 1 | The UE shall report the time stamps of the first transmission/reception during the collection period in the usage information |
| ProSe ReportDataTransmitted | 3 | The UE shall report the amount of data transmitted during the collection period in the usage information with location information |
| ProSe ReportDataReceived | 3 | The UE shall report the amount of data received during the collection period in the usage information with location information |
| ProSe ReportTimeStampsOutOfCoverage | 1 | The UE shall report the time stamps when it went in and out of E-UTRAN coverage during the collection period in the usage information |
| ProSe ReportLocationInCoverage | 1 | The UE shall report the list of locations of the UE when in E-UTRAN coverage during the collection period in the usage information |
| ProSe ReportRadioParameters | 1 | The UE shall report the radio parameters used for ProSe direct communication (i.e. indicator of which radio resources used and radio frequency used) during the reporting period in the usage information |

EFPROSE_GM_DISCOVERY (ProSe Group Member Discovery Parameters)

If service n°10 is "available" in the ProSe Service Table, this file shall be present.

This EF contains the ProSe discovery parameters for public safety, as described in TS 24.334 [57]. This file shall be used only if the ME is authorized as per content of EFAD or received service authorization from the ProSe Function.

User Info ID = 1. Indicates the user information which is sent by the announcing or discoverer or discoveree UE over the air during Group Member Discovery procedures.

Discovery Group ID = 1. Indicates the group ID of the discovery group that the UE belongs to when group member discovery is performed.

Application Layer Group ID = 1.Indicates the Application Layer Group ID identifying an application layer group that the UE belongs to

EFPROSE_RELAY (ProSe Relay Parameters)

If service n°11 is "available" in the ProSe Service Table, this file shall be present.

This EF contains the authorized PLMNs for ProSe UE-to-network relay for public safety, as described in TS 24.334 [57]. This file shall be used only if the ME is authorized as per content of EFAD or received service authorization from the ProSe Function.

PLMN1

File size:	7 Bytes

Default values:	Bytes 1 to 7 (HEX):	A0 05 80 03 00 F1 10

Relay type Value = '02' (the UE is not authorised to act as a ProSe UE-to-network relay, and, the UE is authorised to use a ProSe UE-to-network relay).

EFPROSE_RELAY_DISCOVERY (ProSe Relay Discovery Parameters)

If service n°11 is "available" in the ProSe Service Table, this file shall be present.

This EF contains the ProSe direct discovery parameters when it is used for ProSe UE-to-network relay discovery for public safety, as described in TS 24.334 [37]. This file shall be used only if the ME is authorized as per content of EFAD or received service authorization from the ProSe Function.

User Info ID = 1. Indicates the user information which is sent by the announcing or discoverer or discoveree UE over the air during Group Member Discovery procedures.

Remote UE parameters information

Relay Service Code = 1. Indicates the connectivity service that the ProSe UE-to-network relay provides to public safety applications.

User Info ID of Relay = 2. Indicates the user information of the ProSe UE-to-network relay that the remote UE seeks to discover during ProSe UE-to-network relay discovery procedures.

IP Versions '01' (indicates IPv4)

For Security Content. The value of PSDK is 2.

Bitmask of keys = '07' DUSK, DUCK and DUIK are to be used

DUCK encryption bitmask - all bits set to '1'. 184-bit bitmask which uses bit "1" to mark the positions of the bits for which the DUCK encryption is applied

#### 4.9.3.2	Modified contents of the CSIM Elementary Files

EFCOUNT (Call Count)

The programming of this EF follows default parameter written in 3GPP2 C.S0065-B [48], Annex E.

EFIMSI_M (IMSI_M)

The programming of this EF follows default parameter written in 3GPP2 C.S0065-B [48], Annex E.

EFIMSI_T (IMSI_T)

The programming of this EF follows default parameter written in 3GPP2 C.S0065-B [48], Annex E.

EFTMSI (TMSI)

The programming of this EF follows default parameter written in 3GPP2 C.S0065-B [48], Annex E.

EFAH (Analog Home SID)

The programming of this EF follows default parameter written in 3GPP2 C.S0065-B [48], Annex E.

EFAOP (Analog Operational Parameters)

The programming of this EF is a test house option.

EFALOC (Analog Location	and Registration Indicators)

The programming of this EF is a test house option.

EFCDMAHOME (CDMA Home SID, NID)

Record size:	5 Bytes

Record count:	6

Record 1:

Default values:	Bytes 1 to 2 (HEX):	E8 03		(CDMA Home SID (SIDp): 1000)

Bytes 3 to 4 (HEX):	AE 08		(CDMA Home NID (NIDp): 2222)

Bytes 5 (HEX):		 00			(Band Class: 0 (800 MHz cellular band))

Record 2:

Default values:	Bytes 1 to 2 (HEX):	E8 03		(CDMA Home SID (SIDp): 1000)

Bytes 3 to 4 (HEX):	AE 08		(CDMA Home NID (NIDp): 2222)

Bytes 5 (HEX):		 01			(Band Class: 1 (1.8 to 2.0 GHz PCS band))

Record 3:

Default values:	Bytes 1 to 2 (HEX):	E8 03		(CDMA Home SID (SIDp): 1000)

Bytes 3 to 4 (HEX):	AE 08		(CDMA Home NID (NIDp): 2222)

Bytes 5 (HEX):		 03			(Band Class: 3 (832 to 925 MHz JTACS band))

Record 4:

Default values:	Bytes 1 to 2 (HEX):	E8 03		(CDMA Home SID (SIDp): 1000)

Bytes 3 to 4 (HEX):	AE 08		(CDMA Home NID (NIDp): 2222)

Bytes 5 (HEX):		 04			(Band Class: 4 (1.75 to 1.87 GHz Korean PCS band))

Record 5:

Default values:	Bytes 1 to 2 (HEX):	E8 03		(CDMA Home SID (SIDp): 1000)

Bytes 3 to 4 (HEX):	AE 08		(CDMA Home NID (NIDp): 2222)

Bytes 5 (HEX):		 05			(Band Class: 5 (450 MHz NMT band))

Record 6:

Default values:	Bytes 1 to 2 (HEX):	E8 03		(CDMA Home SID (SIDp): 1000)

Bytes 3 to 4 (HEX):	AE 08		(CDMA Home NID (NIDp): 2222)

Bytes 5 (HEX):		 06			(Band Class: 6 (2 GHz IMT-2000 band))

EFZNREGI (CDMA Zone-Based Registration Indicators)

The programming of this EF follows default parameter written in 3GPP2 C.S0065-B [48], Annex E.

EFSNREGI (CDMA System-Network Registration Indicators)

The programming of this EF is a test house option.

EFDISTREGI (CDMA Distance-Based	Registration Indicators)

The programming of this EF follows default parameter written in 3GPP2 C.S0065-B [48], Annex E.

EFACCOLC (Access Overload Class ACCOLCp)

The 4 LSB of this 1 byte file are der-bit access overload class indicator is derived from the last digit of the associated decimal representation of the IMSI_M vis decimal to binary conversion.

File size:	1 byte

Default value:	Bits 4 to 7 are reserved and set to ‘0000’. Bits 0 to 3 are derived from the last digit of the associated decimal representation of the IMSI_M via decimal to binary conversion.

EFTERM (Call Termination Mode Preferences)

The programming of this EF is a test house option.

EFSSCI (Suggested Slot Cycle Index)

The programming of this EF is a test house option.

EFACP (Analog Channel Preferences)

The programming of this EF is a test house option.

EFPRL (Preferred Roaming List)

File size:	18 Bytes

Default values:	Bytes 1 to 18 (HEX):	00 12 00 00 00 00 40 01 21 00 02 80 00 50 00 00 6E DB

The interpretation of the default values read as follows:

PRL ID – 0

Preferred only – 0

Default roaming indication – 0

Number of Acquisition Records – 1

Number of system records – 1

EFRUIMID (Removable UIMID)

This EF stores a 32-bit electronic identification number (ID) unique to the CSIM or a 32-bit pseudo-UIMID of the CSIM. It is specified by the CSIM manufacturer.

EFCSIM_ST (CSIM Service Table)

Services will be allocated and activated as follows.

| Services |  | Activated | Version |
| --- | --- | --- | --- |
| Service n°1: | Local Phone Book | Option |  |
| Service n°2: | Fixed Dialling Numbers (FDN) | Option |  |
| Service n°3: | Extension 2 | Option |  |
| Service n°4: | Service Dialling Numbers (SDN) | Option |  |
| Service n°5: | Extension3 | Option |  |
| Service n°6: | Short Message Storage | Yes |  |
| Service n°7: | Short Message Parameters (SMP) | Yes |  |
| Service n°8: | HRPD | Yes |  |
| Service n°9: | Service Category Program for BC-SMS | Option |  |
| Service n°10: | CDMA Home Service Provider Name | Yes |  |
| Service n°11: | Data Download via SMS Broadcast (for CCAT) | Option |  |
| Service n°12: | Data Download via SMS-PP (for CCAT) | Option |  |
| Service n°13: | Call Control (for CCAT) | Option |  |
| Service n°14: | 3GPD-SIP | Option |  |
| Service n°15: | 3GPD-MIP | Option |  |
| Service n°16: | AKA | Yes |  |
| Service n°17: | IP-based Location Services (LCS) | Option |  |
| Service n°18: | BCMCS | Option |  |
| Service n°19: | Multimedia Messaging Service (MMS) | Option |  |
| Service n°20: | Extension 8 | Option |  |
| Service n°21: | MMS User Connectivity Parameters | Option |  |
| Service n°22: | Application Authentication | Option |  |
| Service n°23: | Group Identifier Level 1 | Option |  |
| Service n°24: | Group Identifier Level 2 | Option |  |
| Service n°25: | De-Personalization Control Keys | Option |  |
| Service n°26: | Cooperative Network List | Option |  |
| Service n°27: | Outgoing Call Information (OCI) | Option |  |
| Service n°28: | Incoming Call Information (ICI) | Option |  |
| Service n°29: | Extension 5 | Option |  |
| Service n°30: | Multimedia Storage | Option |  |
| Service n°31: | Image (EFIMG) | Option |  |
| Service n°32: | Enabled Services Table | Yes |  |
| Service n°33: | Capability Configuration Parameters (CCP) | Option |  |
| Service n°34: | SF_EUIMID-based EUIMID | Option |  |
| Service n°35: | Messaging and 3GPD Extensions | Option |  |
| Service n°36: | Root Certificates | Option |  |
| Service n°37: | WAP Browser | Option |  |
| Service n°38: | Java | Option |  |
| Service n°39: | Reserved for CDG | No |  |
| Service n°40: | Reserved for CDG | No |  |
| Service n°41: | IPv6 | Option |  |
| Service n°42: | Proactive CSIM (for CCAT) | Option |  |

EFSPC (Service Programming Code)

The programming of this EF follows default parameter written in 3GPP2 C.S0065-B [48], Annex E.

EFOTAPASPC (OTAPA/SPC_Enabled)

The programming of this EF follows default parameter written in 3GPP2 C.S0065-B [48], Annex E.

EFNAMLOCK (NAM_LOCK)

The programming of this EF is a test house option.

EFOTA (OTASP/OTAPA Features)

The programming of this EF is a test house option.

EFSP (Service Preferences)

The programming of this EF is a test house option.

EFESN_MEID_ME (ESN_ME or MEID_ME)

The programming of this EF follows default parameter written in 3GPP2 C.S0065-B [48], Annex E.

EFLI (Language Indication)

The programming of this EF is a test house option.

EFFDN (Fixed Dialling Numbers)

The programming of this EF is a test house option.

EFSMS (Short Messages)

The programming of this EF follows default parameter written in 3GPP2 C.S0065-B [48], Annex E.

EFSMSP (Short Message Service Parameters)

Record size:	12 Bytes

Record count:	1

Record 1:

Default values:	Bytes 1 to 23 (HEX):	00 02 10 02 5D FE FF FF 02 F5 FF FF FF FF FF FF FF FF

08 03 08 01 C0

The interpretation of the default values in this record read as follows:

Teleservice Identifier: CDMA Cellular Messaging Teleservice [CMT-95]

Parameter Indicators: MSG_ENCODING, Validity Period, Bearer Data

Message Encoding: 7-bit ASCII

Validity Period: Indefinite

Bearer Data:

Priority Indicator: Emergency

EFSMSS (SMS Status)

The programming of this EF follows default parameter written in 3GPP2 C.S0065-B [48], Annex E.

EFSSFC (Supplementary Services Feature Code Table)

The programming of this EF is a test house option.

EFSPN (CDMA Home Service Provider Name)

File size:	35 Bytes

Default values:	Bytes 1 to 35 (HEX):	01 02 01 44 65 66 61 75 6C 74 20 53 65 72 76 69 63 65 20 50 72 6F

76 69 64 65 72 20 4E 61 6D 65 FF FF FF

The interpretation of the default values read as follows:

Display Condition: Display of registered system is required

Character Encoding: 7-bit ASCII

Language Indicator: 1 (English)

Service Provider Name: “Default Service Provider Name”

EFUSGIND (UIMID/SF_EUIMID Usage Indicator)

The programming of this EF is a test house option.

EFAD (Administrative Data)

The programming of this EF follows default parameter written in 3GPP2 C.S0065-B [48], Annex E.

EFMDN (Mobile Directory Number)

The programming of this EF is a test house option.

EFMAXPRL (Maximum PRL)

The programming of this EF is a test house option.

EFSPCS (SPC Status)

If EFSPC is set to default value of ’00 00 00’, then EFSPCS shall be set to ‘00’, otherwise it shall be set to ‘01’.

EFECC (Emergency Call Codes)

The programming of this EF follows default parameter written in 3GPP2 C.S0065-B [48], Annex E.

EFME3GPDOPC (ME 3GPD Operation Capability)

The programming of this EF follows default parameter written in 3GPP2 C.S0065-B [48], Annex E.

EF3GPDOPM (3GPD Operation Mode)

The programming of this EF is a test house option.

EFSIPCAP (SimpleIP Capability Parameters)

The programming of this EF is a test house option.

EFMIPCAP (MobileIP Capability Parameters)

The programming of this EF is a test house option.

EFSIPUPP (SimpleIP User Profile Parameters)

The programming of this EF is a test house option.

EFMIPUPP (MobileIP User Profile Parameters)

The programming of this EF is a test house option.

EFSIPSP (SimpleIP Status Parameters)

The programming of this EF is a test house option.

EFMIPSP (MobileIP Status Parameters)

The programming of this EF is a test house option.

EFSIPPAPSS (SimpleIP PAP SS Parameters)

The programming of this EF is a test house option.

EFPUZL (Preferred User Zone List)

The programming of this EF is a test house option.

EFMAXPUZL (Maximum PUZL)

The programming of this EF is a test house option.

EFMECRP (ME-specific Configuration Request Parameters)

The programming of this EF follows default parameter written in 3GPP2 C.S0065-B [48], Annex E.

EFHRPDCAP (HRPD Access Authentication Capability Parameters)

File size:	3 Bytes

Default values:	Bytes 1 to 3 (HEX):	 20 F8 80

The interpretation of the default values read as follows:

Maximum NAI Length: 32

Maximum Length of Shared Secret: 31

Authentication Algorithms: PPP CHAP

EFHRPDUPP (HRPD Access Authentication User Profile Parameters)

File size:	14 Bytes

Default values:	Bytes 1 to 14 (HEX):	0D 0B 61 62 63 40 78 79 7A 2E 63 6F 6D 10

The interpretation of the default values read as follows:

HRPD Profile NAI: “abc@xyz.com”

HRPD Profile Authentication Algorithm: CHAP

EFCSSPR (CUR_SSPR_P_REV)

The programming of this EF follows default parameter written in 3GPP2 C.S0065-B [48], Annex E.

EFATC (Access Terminal Class)

The programming of this EF is a test house option.

EFEPRL (Extended Preferred Roaming List)

File size:	129 Bytes

Default values:	Bytes 1 to 16 (HEX):	00 81 00 00 03 00 01 80 00 04 00 0A 04 01 64 03

Bytes 17 to 32 (HEX):	0B 0B 04 01 64 03 F5 0A 04 0A 58 08 19 0B 04 0A

Bytes 33 to 48 (HEX):	58 0C 97 0A 02 31 13 0B 02 30 C8 71 C0 02 00 10

Bytes 49 to 64 (HEX):	1F 01 00 C8 FF FF 00 80 00 38 E0 08 00 00 80 00

Bytes 65 to 80 (HEX):	71 E0 12 00 10 1F 01 00 C8 FF FF 00 80 00 38 E0

Bytes 81 to 96 (HEX):	18 00 00 80 00 71 C0 22 00 10 1F 01 00 C8 FF FF

Bytes 97 to 112 (HEX):	00 80 80 38 E0 28 00 00 80 80 71 E0 02 00 10 1F

Bytes 113 to 128 (HEX):01 00 C8 FF FF 00 80 80 38 E0 08 00 00 80 80 2B

Byte 129 (HEX):			6B

The interpretation of the default values read as follows:

PRL ID – 0

List Type: IS-683D

Preferred only – 0

Default roaming indication – 0

Number of acquisition records – 6

-		Index: 0, Type: CDMA Generic, Band: 0, Channel: 356 and 779

-	Index: 1, Type: HDR Generic Band: 0, Channel: 356 and 1013

-		Index: 2, Type: CDMA Generic, Band: 1, Channel: 600 and 25

-		Index: 3, Type: HDR Generic, Band: 1, Channel: 600 and 1175

-	Index: 4, Type: CDMA Generic, Band: 6, Channel: 275

-		Index: 5, Type: HDR Generic, Band: 6, Channel: 200

Number of records in the Common Subnet Table – 0

Number of system records – 8

-		Acquisition Index: 1, Type: IS-856, Roam Indicator: 0, Subnet ID: /0, Association Tag: 0

-		Acquisition Index: 3, Type: IS-856, Roam Indicator: 0, Subnet ID: /0, Association Tag: 0

-		Acquisition Index: 5, Type: IS-856, Roam Indicator: 0, Subnet ID: /0, Association Tag: 1

-		Acquisition Index: 1, Type: IS-856, Roam Indicator: 0, Subnet ID: /0, Association Tag: 1

-		Acquisition Index: 0, Type: MCC-MNC-based, Roam Indicator: 0, Subnet ID: MCC-MNC SID/NID, MCC: 001, MNC: 01, SID: 200, NID: 65535100, Association Tag: 0

-		Acquisition Index: 2, Type: MCC-MNC-based, Roam Indicator: 0, Subnet ID: MCC-MNC SID/NID, MCC: 001, MNC: 01, SID: 200, NID: 65535100, Association Tag: 0

-		Acquisition Index: 4, Type: MCC-MNC-based, Roam Indicator: 0, Subnet ID: MCC-MNC SID/NID, MCC: 001, MNC: 01, SID: 200, NID: 65535100, Association Tag: 1

-		Acquisition Index: 0, Type: MCC-MNC-based, Roam Indicator: 0, Subnet ID: MCC-MNC SID/NID, MCC: 001, MNC: 01, SID: 200, NID: 65535, Association Tag: 1

EFMSPL (Multimode system selection System Priority List)

File size:	18 Bytes

Default values:	Bytes 1 to 18 (HEX):	00 0F 01 00 01 01 01 03 07 08 02 0C 00 3E 00 FF FF FF

The interpretation of the default values read as follows:

Current MMSS_P_REV – 1

MLPL Version ID – 1

MLPL Identification – 1

Number of MLPL Records – 1

-	Index: 1, Location Parameter Type Value: default, MSPL Index: 1

EFMLPL (Multimode system selection Location associated Priority List)

File size:	18 Bytes

Default values:	Bytes 1 to 18 (HEX):	00 09 01 00 01 01 01 00 01 FF FF FF FF FF FF FF FF FF

The interpretation of the default values read as follows:

Current MMSS_P_REV – 1

MSPL Version ID – 1

MSPL Block 1:

MSPL Identification – 1,

Number of MSPL ID1 records – 3

-	Index 1: System Type: EUTRA, Priority Class: Home Only, System Priority: 1,
Higher Priority Search Time: 64 minutes, Network Capability Indicator: no Network CAP used

-	Index 2: System Type: cdma2000 HRPD, Priority Class: Home + Preferred, System Priority: 1,
Higher Priority Search Time: 64 minutes, Network Capability Indicator: no Network CAP used

-	Index 3: System Type: cdma2000 AI, Priority Class: Any, System Priority: 1,
Higher Priority Search Time: 64 minutes, Network Capability Indicator: no Network CAP used

EFBCSMScfg (Broadcast Short Message Configuration)

The programming of this EF is a test house option.

EFBCSMSpref (Broadcast Short Message Preference)

The programming of this EF follows default parameter written in 3GPP2 C.S0065-B [48], Annex E.

EFBCSMStable (Broadcast Short Message Table)

The programming of this EF follows default parameter written in 3GPP2 C.S0065-B [48], Annex E.

EFBCSMSP (Broadcast Short Message Parameter)

The programming of this EF follows default parameter written in 3GPP2 C.S0065-B [48], Annex E.

EFBAKPARA (Currently used BAK Parameters)

The programming of this EF is a test house option.

EFUpBAKPARA (Updated BAK Parameters)

The programming of this EF is a test house option.

EFMMSN (MMS Notification)

The programming of this EF follows default parameter written in 3GPP2 C.S0065-B [48], Annex E.

EFEXT8 (Extension 8)

The programming of this EF follows default parameter written in 3GPP2 C.S0065-B [48], Annex E.

EFMMSICP (MMS Issuer Connectivity Parameters)

The programming of this EF follows default parameter written in 3GPP2 C.S0065-B [48], Annex E.

EFMMSUP (MMS User Preferences)

The programming of this EF follows default parameter written in 3GPP2 C.S0065-B [48], Annex E.

EFMMSUCP (MMS User Connectivity Parameters)

The programming of this EF follows default parameter written in 3GPP2 C.S0065-B [48], Annex E.

EFAuthCapability (Authentication Capability)

The programming of this EF follows default parameter written in 3GPP2 C.S0065-B [48], Annex E.

EF3GCIK (3G Cipher and Integrity Keys)

The programming of this EF is a test house option.

EFDCK (De-Personalization Control Keys)

The programming of this EF is a test house option.

EFGID1 (Group Identifier Level 1)

The programming of this EF is a test house option.

EFGID2 (Group Identifier Level 2)

The programming of this EF is a test house option.

EFCDMACNL (CDMA Co-operative Network List)

The programming of this EF is a test house option.

EFHOME_TAG (Home System Tag)

The programming of this EF is a test house option.

EFGROUP_TAG (Group Tag List)

The programming of this EF is a test house option.

EFSPECIFIC_TAG (Specific Tag List)

The programming of this EF is a test house option.

EFCALL_PROMPT (Call Prompt List)

The programming of this EF is a test house option.

EFSF_EUIMID (Short Form	EUIMID)

Specified by CSIM Manufacturer.

EFEST (Enabled Service Table)

The programming of this EF is a test house option.

EFHiddenKey (Key for hidden phone book entries)

The programming of this EF is a test house option.

EFLCSVER (LCS Protocol Version)

The programming of this EF is a test house option.

EFLCSCP (LCS Connectivity Parameter)

The programming of this EF is a test house option.

EFSDN (Service Dialling	Numbers)

The programming of this EF is a test house option.

EFEXT2(Extension2)

The programming of this EF is a test house option.

EFEXT3(Extension3)

The programming of this EF is a test house option.

EFICI (Incoming Call Information)

The programming of this EF is a test house option.

EFOCI (Outgoing Call Information)

The programming of this EF is a test house option.

EFEXT5 (Extension 5)

The programming of this EF is a test house option.

EFCCP2 (Capability Configuration Parameters 2)

The programming of this EF is a test house option.

EFAppLabels (Application Labels)

The programming of this EF is a test house option.

EFModel (Device Model Information)

The programming of this EF follows default parameter written in 3GPP2 C.S0065-B [48], Annex E.

EFRC (Root Certificates)

The programming of this EF is a test house option.

EFSMSCAP (SMS Capabilities)

File size:	4 Bytes

Default values:	Bytes 1 to 4 (HEX):	 3C 08 07 01

The interpretation of the default values read as follows:

SMS Retry Period: 60 seconds

SMS Retry Interval: 8 seconds

SMS Flags:

Send On Access: True

Send on Traffic: True

Send as Standard EMS: True

SMS Preferred Service Option: SO 6

EFMIPFlags (MobileIP Flags)

The programming of this EF is a test house option.

EF3GPDUPPExt (3GPD User Profile Parameters Extension)

The programming of this EF is a test house option.

EFIPV6CAP (IPv6 Capabilities)

The programming of this EF is a test house option.

EFTCPConfig (TCp Configurations)

The programming of this EF is a test house option.

EFDGC (Data Generic Configurations)

The programming of this EF is a test house option.

EFWAPBrowserCP (WAP Browser Connectivity Parameters)

The programming of this EF is a test house option.

EFWAPBrowserBM (WAP Browser Bookmarks)

The programming of this EF follows default parameter written in 3GPP2 C.S0065-B [48], Annex E.

EFMMSConfig (MMS Configuration)

The programming of this EF is a test house option.

EFJDL (Java Download URL)

The programming of this EF is a test house option.

#### 4.9.3.3	Modified contents of the USIM Elementary Files and additional USIM Elements files at the MCPTT level

EFUST (USIM Service Table)

| Services | Description | Activated | Version |
| --- | --- | --- | --- |
| Service n°109 | MCPTT | Optional |  |
| Note: Only the relevant MCPTT related services indicated. |  |  |  |

EFMST (MCPTT Service Table)

If service n°109 is "available" in the USIM Service Table or service n°15 is "available" in the ISIM Service Table, this file shall be present. This EF indicates the coding of the MCPTT management objects and which MCPTT services are available. If a service is not indicated as available in the MCPTT Service Table, the ME shall not select this service.

Coding of the MCPTT management objects = '00' (XML format).

| Services | Description | Activated | Version |
| --- | --- | --- | --- |
| Service n°1: | UE configuration data |  |  |
| Service n°2: | User configuration data |  |  |
| Service n°3: | Group configuration data |  |  |
| Service n°4: | Service configuration data |  |  |

EFMCPTT_CONFIG (MCPTT configuration data)

If service n°1, 2, 3 or 4 is "available" in the MCPTT Service Table, this file shall be present.

Encoded in XML format (as specified in the MCPTT Service Table).

| MCPTT configuration data objects | Tag Values | Condition |
| --- | --- | --- |
| MCPTT UE configuration data | '80' | Shall be present if service n°1 is "available" in the MCPTT service table |
| MCPTT User configuration data | '81' | Shall be present if service n°2 is "available" in the MCPTT service table |
| MCPTT Group configuration data | '82' | Shall be present if service n°3 is "available" in the MCPTT service table |
| MCPTT Service configuration data | '83' | Shall be present if service n°4 is "available" in the MCPTT service table |
| Note: MCPTT configuration data objects, as specified in 3GPP TS 24.483 [66] Annex B.2, Annex B3, Annex B.4 and Annex B.5. |  |  |

#### 4.9.3.4	Modified contents of the USIM Elementary Files and additional USIM Elements files at the DFV2X level

EFUST (USIM Service Table):

| Services |  | Activated | Version |
| --- | --- | --- | --- |
| Service n°119 | V2X | Optional |  |
| Note: Only V2X related services indicated. |  |  |  |

EFVST (V2X Service Table)

If service n°119 is "available" in the USIM Service Table, this file shall be present. This EF indicates the coding of the V2X management objects and which V2X services are available.

File size:	2 Bytes

Default values:	Bytes 1 to 2 (HEX):	00 01

Coding of the V2X management objects is XML format described in TS 24.385 [97].

Service n°1 V2X configuration data is supported.

EFV2X_CONFIG (V2X configuration data)

This EF contains zero, one or more V2X configuration data objects, as specified in TS 24.385 [97].

File size:	1+X Bytes

Default values:	Bytes 1  (HEX):	 80

Bytes 2 to X (HEX):	Coding result of V2X management objects

Coding of the V2X management objects is XML format described in TS 24.385.

V2X configuration data object tag:

| V2X configuration data objects | Tag Values | Condition |
| --- | --- | --- |
| V2X configuration data | '80' | Shall be present if service n°1 is "available" in the V2X service table |
| Note: V2X configuration data objects, as specified in 4.10.2. |  |  |

#### 4.9.3.5	Modified contents of the Elementary Files for eCall over IMS

For the eCall over IMS test cases EFs shall be extended as follows:

-	Table 4.9.3.5-1 specifies the EF extensions for test cases for eCall capable supporting UEs,

-	Table 4.9.3.5-2 specifies the EF extensions for test cases for eCall only supporting UEs.

Table 4.9.3.5-1: EF extensions eCall capable UEs

| EF field | Value |
| --- | --- |
| EFUST | Service n°4 Service Dialling Numbers (SDN), Service n°99 URI support by UICC, Service n°89 eCall Data and Service n°112 eCall Data over IMS are available |
| EFEST | Services n°1 Fixed Dialling Numbers (FDN) is disabled |
| EFSDN | Two entries of SDNs, eCall Test Number (123456) and eCall reconfiguration number (345678) |
| EFSDNURI | Two entries of SDNs, eCall Test Number (tel:123456) and eCall reconfiguration number (tel:345678) |

Table 4.9.3.5-2: EF extensions for eCall only UEs

| EF field | Value |
| --- | --- |
| EFUST | Service n°2 Fixed Dialling Numbers (FDN), Service n°99 URI support by UICC, Service n°89 eCall Data and Service n°112 eCall Data over IMS are available |
| EFEST | Service n°1 Fixed Dialling Numbers (FDN) is enabled |
| EFFDN | Two entries of FDNs, eCall Test Number (123456) and eCall reconfiguration number (345678) |
| EFFDNURI | Two entries of FDNs, eCall Test Number (tel:123456) and eCall reconfiguration number (tel:345678) |

## 4.10	V2X reference configuration

This clause contains the default reference pre-configurations, which apply to all V2X test cases unless otherwise specified.

### 4.10.1	Reference configuration for V2X Sidelink Communication

ROHC compression is not applied in test cases simulating in coverage and out-of-coverage scenarios.

Ciphering is not configured on SLRB.

#### 4.10.1.1	V2X Sidelink Communication Preconfiguration for out-of-network coverage operation

The IE SL-V2X-Preconfiguration contains the resource pool information for V2X sidelink communication.

Table 4.10.1.1-1: SL-V2X-Preconfiguration

| Derivation Path: 36.331 clause 9.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-V2X-Preconfiguration-r14 ::= SEQUENCE { |  |  |  |
| v2x-PreconfigFreqList-r14 SEQUENCE (SIZE (1..maxFreqV2X-r14)) OF SEQUENCE { |  |  |  |
| SL-V2X-PreconfigFreqInfo-r14[1] | SL-V2X-PreconfigFreqInfo-r14-DEFAULT |  |  |
| } |  |  |  |
| anchorCarrierFreqList-r14 | Not present |  |  |
| cbr-PreconfigList-r14 | Not present |  |  |
| } |  |  |  |

Table 4.10.1.1-2: SL-V2X-Preconfiguration for V2X sidelink communication with MCC

| Derivation Path: 36.331 clause 9.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-V2X-Preconfiguration-r14 ::= SEQUENCE { |  |  |  |
| v2x-PreconfigFreqList-r14 SEQUENCE (SIZE (1..maxFreqV2X-r14)) OF SEQUENCE { | 2 entries of SL-V2X-PreconfigFreqInfo-r14 |  |  |
| SL-V2X-PreconfigFreqInfo-r14 [1] | SL-V2X-PreconfigFreqInfo-r14-DEFAULT |  |  |
| SL-V2X-PreconfigFreqInfo-r14 [2] | SL-V2X-PreconfigFreqInfo-r14-DEFAULT |  |  |
| } |  |  |  |
| anchorCarrierFreqList-r14 | Not present |  |  |
| cbr-PreconfigList-r14 | Not present |  |  |
| } |  |  |  |

### 4.10.2	V2X management data object configuration

V2X management data object is specified in 3GPP TS 24.385. This subclause provides the default content of V2X management object for conformance testing.

The V2X management data object can be configured in USIM or pre-configured in ME.

| Derivation Path: TS 24.385 clause 5 |  |  |
| --- | --- | --- |
| V2X management objects | MO Values | Comment |
| <X> | N/A |  |
| Name | Default Management Object for Conformance Testing |  |
| V2XoverPC5 | N/A |  |
| Expiration | BIN‘0000000000000000001001111000110100000000’ | 31st January 2012 00:31:00 (GPS time) (UTC time: TBD) |
| ServedByEUTRAN | N/A |  |
| AuthorizedPLMNs | N/A |  |
| <X> | N/A |  |
| PLMN | Set to the same Mobile Country Code and Mobile Network Code stored in EFIMSI on the test USIM card |  |
| NotServedByEUTRAN | N/A |  |
| Authorized | 1 | Indicates that the UE is authorized to use V2X communication over PC5 when the UE is not served by E-UTRAN for V2X communication |
| RadioParameters | N/A |  |
| <X> | N/A |  |
| RadioParametersContents | Set as defined in SL-V2X-Preconfiguration in subclause 4.10 |  |
| GeographicalArea | N/A |  |
| Polygon | N/A |  |
| <X> | N/A |  |
| Coordinates | N/A |  |
| <X> | N/A |  |
| Latitude | Set as defined in GNSS scenario used by specific test case |  |
| Longitude | Set as defined in GNSS scenario used by specific test case |  |
| OperatorManaged | 0 | Indicates that the radio parameters are "non-operator managed" |
| AuthorizedV2XServiceList | N/A |  |
| <X> | N/A |  |
| V2XServiceIdentifier | HEX ‘3000’ |  |
| DestinationLayer2ID | HEX ‘0000AA’ | Same as default Destination Layer2ID defined in Table 4.7I-2 |
| PPPPtoPDBMappingRule | N/A |  |
| <X> | N/A | 1st entry of PPPPtoPDBMappingRule |
| ProSePerPacketPriority | 1 |  |
| PacketDelayBudget | 100 |  |
| <X> | N/A | 2nd entry of PPPPtoPDBMappingRule |
| ProSePerPacketPriority | 2 |  |
| PacketDelayBudget | 100 |  |
| <X> | N/A | 3rd entry of PPPPtoPDBMappingRule |
| ProSePerPacketPriority | 3 |  |
| PacketDelayBudget | 100 |  |
| <X> | N/A | 4th entry of PPPPtoPDBMappingRule |
| ProSePerPacketPriority | 4 |  |
| PacketDelayBudget | 100 |  |
| <X> | N/A | 5th entry of PPPPtoPDBMappingRule |
| ProSePerPacketPriority | 5 |  |
| PacketDelayBudget | 100 |  |
| <X> | N/A | 6th entry of PPPPtoPDBMappingRule |
| ProSePerPacketPriority | 6 |  |
| PacketDelayBudget | 100 |  |
| <X> | N/A | 7th entry of PPPPtoPDBMappingRule |
| ProSePerPacketPriority | 7 |  |
| PacketDelayBudget | 100 |  |
| <X> | N/A | 8th entry of PPPPtoPDBMappingRule |
| ProSePerPacketPriority | 8 |  |
| PacketDelayBudget | 100 |  |
| DefaultDestinationLayer2ID | Not present |  |
| PrivacyConfig | Not present |  |
| V2XServiceIDtoV2XFrequencyMappingRule | Not present |  |
| V2XoverLTEUu |  |  |
| Expiration | BIN‘0000000000000000001001111000110100000000’ | 31st January 2012 00:31:00 (GPS time) (UTC time: TBD) |
| AuthorizedPLMNs | N/A |  |
| <X> | N/A |  |
| PLMN | Set to the same Mobile Country Code and Mobile Network Code stored in EFIMSI on the test USIM card |  |
| V2XASMBMSConfig | N/A |  |
| TMGI | MBMS Service ID set to HEX ‘000000’ MCC and MNC are the same Mobile Country Code and Mobile Network Code stored in EFIMSI on the test USIM card |  |
| SAIList | N/A |  |
| <X> | N/A |  |
| SAI | 0 |  |
| Frequency | f1 as per table 6.2.3.3-2 using the same band as cell1 |  |
| SDPInV2XASMBMSConfig | vnd.3gpp.v2x |  |
| V2XServiceIdentifierUnrelated | N/A |  |
| V2XMBMSConfigs | N/A |  |
| <X> | N/A |  |
| V2XMBMSConfig | N/A |  |
| TMGI | MBMS Service ID set to HEX ‘000000’ MCC and MNC are the same Mobile Country Code and Mobile Network Code stored in EFIMSI on the test USIM card |  |
| SAIList | N/A |  |
| <X> | N/A |  |
| SAI | 0 |  |
| Frequency | f1 as per table 6.2.3.3-2 using the same band as cell1 |  |
| SDPInV2XASMBMSConfig | vnd.3gpp.v2x |  |
| V2XServiceIdentifierRelated | N/A |  |
| AuthorizedV2XServiceList | N/A |  |
| <X> | N/A |  |
| V2XServiceIdentifier | HEX ‘3000’ |  |
| V2XMBMSConfigs | N/A |  |
| TMGI | MBMS Service ID set to HEX ‘000000’ MCC and MNC are the same Mobile Country Code and Mobile Network Code stored in EFIMSI on the test USIM card |  |
| SAIList | N/A |  |
| <X> | N/A |  |
| SAI | 0 |  |
| Frequency | f1 as per table 6.2.3.3-2 using the same band as cell1 |  |
| SDPInV2XASMBMSConfig | vnd.3gpp.v2x |  |

## 4.11	GNSS Requirements for V2X and MCS testing

### 4.11.1	General

This clause defines the GNSS scenarios and requirements which apply for all V2X and MCS test cases that require simulated GNSS signals, unless otherwise specified.

The term SV ID used in this clause is defined as the satellite PRN for GPS and Modernized GPS, as Code Number for Galileo, as the satellite Slot Number for GLONASS and as the Ranging Code Number for BDS.

### 4.11.2	GNSS Scenarios

The following GNSS scenarios shall be used.

-	Yuma / Rinex Almanac data: the required file(s) available in the GNSS data sig zip file specified in TS 37.571-5 [68] Annex B are given in Table 4.11.2-1.

Table 4.11.2-1: Yuma / Rinex Almanac data files for V2X and MCS testing

| GNSS supported by UE | Yuma / Rinex file(s) (1) |
| --- | --- |
| GPS | Sig GNSS 1-3 Yuma.txt |
| GLONASS | Sig GNSS 1-1 AGL.txt |
| Galileo | Sig GNSS 1-2 Yuma.txt |
| BDS | FFS |
| Note 1: Where the UE supports more than one GNSS then all the relevant Yuma / Rinex data files are used |  |

-	UE location(s) and motion:

Latitude: the simulated latitude(s) are given in Table 4.11.2-3

Longitude: the simulated longitude(s) are given in Table 4.11.2-3

Height: the simulated height is 30m

Motion: the simulated motion(s) are given in Table 4.11.2-2

Table 4.11.2-2: UE location(s) and motion(s) for V2X and MCS testing

| Scenario number and description | Step # | Action (Location details given in Table 4.11.2-3) | Notes |
| --- | --- | --- | --- |
| Scenario #1: static in Geographical area #1 Note 1 | 1 | Static at location #1 |  |
| Scenario #2: move from inside Geographical area #1 to outside Geographical area #1 Note 1 | 1 | Static at location #1 |  |
|  | 2 | Trigger from test case to move in a straight line at 15m/s from current location to next location | Simulation leaves Geographical area #1 after 905 m, 60s after the trigger. An additional 1s is added to allow for UE position accuracy of +/- 15m. An additional 10s is added to allow for UE position update. Total time 71s |
|  | 3 | Static at location #2 |  |
| Scenario #3: move from zone id0 to zone id1 to zone id3 to zone id2 | 1 | Static at location #3 |  |
|  | 2 | Trigger from test case to move in a straight line at 2m/s from current location to next location | Simulation crosses the boundary between zone id0 and zone id1 47s after the trigger. An additional 7.5s is added to allow for UE position accuracy of +/- 15m. An additional 10s is added to allow for UE position update. Total time 64.5s |
|  | 3 | Static at location #4 |  |
|  | 4 | Trigger from test case to move in a straight line at 2m/s from current location to next location | Simulation crosses the boundary between zone id1 and zone id3 42s after the trigger. An additional 7.5s is added to allow for UE position accuracy of +/- 15m. An additional 10s is added to allow for UE position update. Total time 59.5s |
|  | 5 | Static at location #5 |  |
|  | 6 | Trigger from test case to move in a straight line at 2m/s from current location to next location | Simulation crosses the boundary between zone id3 and zone id2 47s after the trigger. An additional 7.5s is added to allow for UE position accuracy of +/- 15m. An additional 10s is added to allow for UE position update. Total time 64.5s |
|  | 7 | Static at location #6 |  |
| Scenario #4: move from inside Geographical area #1 to location #7 and then location #8 inside Geographical area #1 Note 1 | 1 | Static at location #1 |  |
|  | 2 | Trigger from test case to move in a straight line at 10m/s from current location to next location | Simulation takes approximately 10 secs to move from location #1 to location #7 |
|  | 3 | Static at location #7 |  |
|  | 4 | Trigger from test case to move in a straight line at 10m/s from current location to next location | Simulation takes approximately 10 secs to move from location #7 to location #8 |
|  | 5 | Static at location #8 |  |
| Note 1: Geographical area #1 is defined in clause 4.9.3.1 and is defined by three points with the following coordinates: Point 1: latitude: 35.753056, longitude: 139.689167 Point 2: latitude: 35.735278, longitude: 139.689167 Point 3: latitude: 35.744167, longitude: 139.709167 |  |  |  |

Table 4.11.2-3: Location descriptions for V2X and MCS testing

| Location number | Description for information only | Latitude (degrees) | Longitude (degrees) |
| --- | --- | --- | --- |
| #1 | Approximate centre of Geographical area #1 | 35.74428 | 139.69916 |
| #2 | Approximately 1720m due west of location #1, outside Geographical area #1 | 35.74428 | 139.68017 |
| #3 | Centre of zone id0 inside Geographical area #1, see Table 4.11.2-4 and Figure 4.11.2-1 | 35.74478 | 139.70333 |
| #4 | Centre of zone id1 inside Geographical area #1, see Table 4.11.2-4 and Figure 4.11.2-1 | 35.74478 | 139.70540 |
| #5 | Centre of zone id3 inside Geographical area #1, see Table 4.11.2-4 and Figure 4.11.2-1 | 35.74523 | 139.70716 |
| #6 | Centre of zone id2 inside Geographical area #1, see Table 4.11.2-4 and Figure 4.11.2-1 | 35.74523 | 139.70508 |
| #7 | Approximately 100m due west of location #1, inside Geographical area #1 | 35.74428 | 139.69806 |
| #8 | Approximately 100m due west of location #7, inside Geographical area #1 | 35.74428 | 139.69695 |

Table 4.11.2-4: Zone id calculations for V2X testing

| Description | Latitude (degrees) | Longitude (degrees) | Long distance from 0,0 in m (x) Note 1 | Lat distance from 0,0 in m (y) Note 1 | Values for zone_id calculation (Nx, Ny = 2, L = 100, W = 50 Note 2) Note 3 |
| --- | --- | --- | --- | --- | --- |
| Centre of zone id0 | 35.74478 | 139.70333 | 11061850 | 3957225 | y1= 0, x1= 0 |
| Centre of zone id1 | 35.74478 | 139.70540 | 11061950 | 3957225 | y1= 0, x1= 1 |
| Centre of zone id2 | 35.74523 | 139.70508 | 11061850 | 3957275 | y1= 1, x1= 0 |
| Centre of zone id3 | 35.74523 | 139.70716 | 11061950 | 3957275 | y1= 1, x1= 1 |
| Note 1: all distances are +/- 1m Note 2: zone length (longitude) is set to 100m, zone width (latitude) is set to 50m in accordance with TS 36.523-1 [18] Table 24.1.8.3.3-1 and Table 24.1.9.3.3-1, this allows for the accuracy with which a UE can define its position which is assumed to be +/- 15m. Zone id longitude modulus and zone id latitude modulus are set to 2 in accordance with TS 36.523-1 [18] Table 24.1.8.3.3-1 and Table 24.1.9.3.3-1. Note 3: zone ids are calculated according to TS 36.331 [17] clause 5.10.13.2 |  |  |  |  |  |

Figure 4.11.2-1: Zones and motions for scenario #3 for V2X testing (not to scale)

-	Nominal start time:

1st January 2012 00:31:00 (GPS time) (UTC time: TBD).

-	Visible satellites to be simulated are given in Table 4.11.2-5 and are above 15 degrees elevation with respect to the UE. These satellites have been selected to give a reasonable HDOP for the duration of the test.

Table 4.11.2-5: Satellites to be simulated for V2X and MCS testing

| GNSS supported by UE | SV IDs of Satellites to be simulated (1) |
| --- | --- |
| GPS | 1, 11, 17, 20, 23, 28 |
| GLONASS | 3, 4, 9, 10, 18, 20 |
| Galileo | 5, 10, 11, 18, 19, 20 |
| BDS | FFS |
| Note 1: Where the UE supports more than one GNSS then all the relevant satellites are simulated |  |

-	The levels of the simulated satellites are given in Table 4.11.2-6 and shall be generated with an accuracy of +/- [1] dB. These conditions are defined for when there is no GNSS assistance data available at the UE and are specified in TS 36.133 [39] clause B.6.

Table 4.11.2-6: GNSS Reference Signal Power Requirements

| System | Parameters | Unit | Value |
| --- | --- | --- | --- |
|  | Number of generated satellites per system | - | 6 |
| GPS(1) | Reference signal power level for all satellites | dBm | -128.5 |
| Galileo | Reference signal power level for all satellites | dBm | -127 |
| GLONASS | Reference signal power level for all satellites | dBm | -131 |
| BDS | Reference signal power level for all satellites | dBm | -133 |
| NOTE 1: "GPS" here means GPS L1 C/A, Modernized GPS, or both, dependent on UE capabilities. NOTE 2: The DUT UE shall support at least one system and will be configured for the supported system(s). |  |  |  |

-	Ionospheric model: simulated values are given in Tables 4.11.2-7 and 4.11.2-8.

Table 4.11.2-7: Klobuchar ionospheric model for GPS or GLONASS or BDS if supported by the UE

| Model element | Units | Value/remark |
| --- | --- | --- |
| alfa0 | seconds | 4.6566129 10E-9 |
| alfa1 | sec/semi-circle | 1.4901161 10E-8 |
| alfa2 | sec/(semi-circle)2 | -5.96046 10E-8 |
| alfa3 | sec/(semi-circle)3 | -5.96046 10E-8 |
| beta0 | seconds | 79872 |
| beta1 | sec/semi-circle | 65536 |
| beta2 | sec/(semi-circle)2 | -65536 |
| beta3 | sec/(semi-circle)3 | -393216 |

Table 4.11.2-8: neQuick ionospheric model for Galileo if supported by the UE

| Model Element | Units | Value/remark |
| --- | --- | --- |
| ai0 | solar flux unit | 64.4 |
| ai1 | solar flux unit/degree | 0 |
| ai2 | solar flux unit/degree2 | 0 |

-	Tropospheric model: STANAG with SRI equal to 324.8, as defined in STANAG 4294 [70].

## 4.12	GNSS Requirements for aerial testing

### 4.12.1	General

This clause defines the GNSS scenarios and requirements which apply for all aerial test cases that require simulated GNSS signals, unless otherwise specified.

The term SV ID used in this clause is defined as the satellite PRN for GPS and Modernized GPS, as Code Number for Galileo, as the satellite Slot Number for GLONASS and as the Ranging Code Number for BDS.

### 4.12.2	GNSS Scenarios

The following GNSS scenarios shall be used.

-	Yuma / Rinex Almanac data: the required file(s) available in the GNSS data sig zip file specified in TS 37.571-5 [68] Annex B are given in Table 4.12.2-0.

Table 4.12.2-0: Yuma / Rinex Almanac data files for Aerial testing

| GNSS supported by UE | Yuma / Rinex file(s) (1) |
| --- | --- |
| GPS | globalAlmanac_gpsYuma.txt |
| GLONASS | globalAlmanac_glonass.agl |
| Galileo | globalAlmanac_galileo.xml |
| BDS | FFS |
| Note 1: Where the UE supports more than one GNSS then all the relevant Yuma / Rinex data files are used |  |

-	UE location(s) and motion:

Latitude: the simulated latitude is given in Table 4.12.2-2

Longitude: the simulated longitude is given in Table 4.12.2-2

Height: the simulated heights above sea level are given in Table 4.12.2-2

Motion: the simulated motion(s) are given in Table 4.12.2-1

Table 4.12.2-1: UE locations and motions for aerial testing

| Scenario number and description | Step # | Action (Location details given in Table 4.12.2-3) | Notes |
| --- | --- | --- | --- |
| Scenario #1: move from location #1 to location #2 to location #3 | 1 | Static at location #1 |  |
|  | 2 | Trigger from test case to move in a straight line at 5m/s from current location to next location |  |
|  | 3 | Static at location #2 |  |
|  | 4 | Trigger from test case to move in a straight line at 5m/s from current location to next location | Simulation will take 40 seconds to move between locations given the nominal values specified in the test case. An additional 10 seconds is added to allow for UE position update. Total time 50 seconds. |
|  | 5 | Static at location #3 |  |

Table 4.12.2-2: Location descriptions for aerial testing

| Location number | Latitude (degrees) | Longitude (degrees) | Height above sea level (m) | Notes |
| --- | --- | --- | --- | --- |
| #1 | 43.308318 | -5.697122 | See test case |  |
| #2 | 43.308318 | -5.697122 | See test case |  |
| #3 | 43.308318 | -5.697122 | See test case |  |

-	Nominal start time:

Current date at 07:31:00 (GPS time).

-	Visible satellites to be simulated are given in Table 4.12.2-3 and are above 15 degrees elevation with respect to the UE. These satellites have been selected to give a reasonable HDOP for the duration of the test.

Table 4.12.2-3: Satellites to be simulated for Aerial testing

| GNSS supported by UE | SV IDs of Satellites to be simulated (1) |
| --- | --- |
| GPS | 2, 12, 14, 24, 25, 29 |
| GLONASS | 4, 5, 13, 14, 15, 23 |
| Galileo | 2, 7, 8, 11, 19, 30 |
| BDS | 13, 15, 19, 27, 28, 30 |
| Note 1: Where the UE supports more than one GNSS then all the relevant satellites are simulated |  |

-	The levels of the simulated satellites are given in Table 4.11.2-6 and shall be generated with an accuracy of +/- 1 dB.

-	Ionospheric model: simulated values are given in Tables 4.11.2-7 and 4.11.2-8.

-	Tropospheric model: STANAG with SRI equal to 324.8, as defined in STANAG 4294 [70].