## 6.5	Transmit signal quality

Editor's note:	The test cases for transmit signal quality: frequency error, EVM, carrier leakage, IBE, EVM equalizer spectrum flatness are complete, except the following aspect is not determined:

-	Reference signal EVM and PRACH EVM minimum requirements from the core specification are still in brackets.

In this clause a multitude of results are derived, all using one common algorithm returning these results: Global In-Channels TX-Test (Annex E). Each sub clause of this clause contains a procedure and test requirements described for a specific measurement. If all relevant test parameters in different sub clauses are the same, then the results, returned by the Global In-Channel TX-Test, may be used across the applicable sub clauses.

For UE category M1, carrier centre frequency can be either at the centre of channel bandwidth or the 6RB narrowband assigned for transmission. TE shall measure frequency error, EVM, carrier leakage, In-band emissions and EVM equalizer spectrum flatness using Global In-Channels TX-Test (Annex E) according to the UE’s declaration on the position of carrier centre frequency.

### 6.5.1	Frequency Error

#### 6.5.1.1	Test purpose

This test verifies the ability of both, the receiver and the transmitter, to process frequency correctly.

Receiver: to extract the correct frequency from the stimulus signal, offered by the System simulator, under ideal propagation conditions and low level.

Transmitter: to derive the correct modulated carrier frequency from the results, gained by the receiver.

#### 6.5.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 8 and forward.

#### 6.5.1.3	Minimum conformance requirements

The UE modulated carrier frequency shall be accurate to within ±0.1 PPM observed over a period of one time slot (0.5ms) compared to the carrier frequency received from the E-UTRA Node B.

The normative reference for this requirement is TS 36.101 clause 6.5.1

#### 6.5.1.4	Test description

##### 6.5.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.5.1.4.1-1. The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annexes A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Lowest, 5MHz, Highest |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |  |
| Ch BW | Mod'n | RB allocation |  | Mod'n | RB allocation |  |
|  |  | FDD | TDD |  | FDD | TDD |
| 1.4MHz | QPSK | 6 | 6 | QPSK | 6 | 6 |
| 3MHz | QPSK | 15 | 15 | QPSK | 15 | 15 |
| 5MHz | QPSK | 25 | 25 | QPSK | 25 | 25 |
| 5MHz | QPSK | 25 | N/A | QPSK | 20 | N/A |
| 5MHz | QPSK | 25 | N/A | QPSK | 15 | N/A |
| 5MHz | QPSK | 25 | N/A | QPSK | 53 | N/A |
| 10MHz | QPSK | 50 | 50 | QPSK | 50 | 50 |
| 10MHz | QPSK | 50 | N/A | QPSK | 25 | N/A |
| 10MHz | QPSK | 50 | N/A | QPSK | 20 | N/A |
| 10MHz | QPSK | 50 | N/A | QPSK | 15 | N/A |
| 15MHz | QPSK | 75 | 75 | QPSK | 75 | 75 |
| 15MHz | QPSK | 75 | N/A | QPSK | 50 | N/A |
| 15MHz | QPSK | 75 | N/A | QPSK | 25 | N/A |
| 15MHz | QPSK | 75 | N/A | QPSK | 20 | N/A |
| 20MHz | QPSK | 100 | 100 | QPSK | 100 | 100 |
| 20MHz | QPSK | 100 | N/A | QPSK | 75 | N/A |
| 20MHz | QPSK | 100 | N/A | QPSK | 50 | N/A |
| 20MHz | QPSK | 100 | N/A | QPSK | 25 | N/A |
| 20MHz | QPSK | 100 | N/A | QPSK | 20 | N/A |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: Depending on E-UTRA band, only the appropriate Uplink RB allocation value according to table 7.3.3-2 is tested per Test Channel Bandwidth. Note 3: Applicable only to E-UTRA FDD Band 31. The UL resource blocks shall be located at RBstart 10 (according to Table 7.3.3-2). |  |  |  |  |  |  |

1.	Connect the SS to the UE to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.3.

2.	The parameter settings for the cell are set up according to TS 36.508[7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL and DL Reference Measurement channels are set according to Table 6.5.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.5.1.4.3.

##### 6.5.1.4.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1A for C_RNTI to transmit the DL RMC according to Table 6.5.1.4.1-1. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.5.1.4.1-1, since the UE has no payload data to send, the UE transmit uplink MAC padding bits on the UL RMC

3.	Set the Downlink signal level to the appropriate REFSENS value defined in Table 7.3.5-1. Send continuously uplink power control "up" commands to the UE in every uplink scheduling information to the UE so that the UE transmits at PUMAX level for the duration of the test.

4.	Measure the Frequency Error using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test.

##### 6.5.1.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the exceptions in subclause 7.3.4.3 and Table 7.3.3-3.

#### 6.5.1.5	Test requirement

The 20 frequency error Δf results must fulfil the test requirement:

|Δf| ≤ (0.1 PPM + 15 Hz)

### 6.5.1A	Frequency error for CA

#### 6.5.1A.0	Minimum conformance requirements

For inter-band carrier aggregation with uplink assigned to two E-UTRA bands, the frequency error requirements defined in subclause 6.5.1 shall apply on each component carrier with both component carriers active.

For intra-band contiguous carrier aggregation the UE modulated carrier frequencies per band shall be accurate to within ±0.1 PPM observed over a period of one timeslot compared to the carrier frequency of primary component carrier received from the E-UTRA in the corresponding band.

For intra-band non-contiguous carrier aggregation the requirements in Section 6.5.1 applies per component carrier.

The normative reference for this requirement is TS 36.101[2] clause 6.5.1A.

#### 6.5.1A.1	Frequency error for CA (intra-band contiguous DL CA and UL CA)

##### 6.5.1A.1.1	Test purpose

This test verifies the ability of both, receiver and the transmitter, to process frequency for intra-band CA correctly.

Receiver: to extract the correct frequency from the stimulus signal, offered by the System simulator, under ideal propagation conditions and low level.

Transmitter: to derive the correct modulated carrier frequency from the results, gained by the receiver.

##### 6.5.1A.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 10 and forward that support intra-band contiguous DL CA and UL CA.

##### 6.5.1A.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5.1A.0.

##### 6.5.1A.1.4	Test description

6.5.1A.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on E-UTRA CA configurations specified in table 5.4.2A.1-1. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 6.5.1A.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.1A.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  | Low range, High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 3) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | UL Allocation |  |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation | CC MOD | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 25 | 25 | N/A | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 25 | 25 |  | QPSK | 50 | P_25@0 | S_25@0 | - | - |
| 25 | 50 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 25 | 50 |  | QPSK | 75 | P_25@0 | S_50@0 | - | - |
| 50 | 25 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 50 | 25 |  | QPSK | 75 | P_50@0 | S_25@0 | - | - |
| 50 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 50 | 50 |  | QPSK | 100 | P_50@0 | S_50@0 | - | - |
| 75 | 25 |  | QPSK | 75 | P_75@0 | S_0@0 | - | - |
| 75 | 25 |  | QPSK | 100 | P_75@0 | S_25@0 | - | - |
| 75 | 75 |  | QPSK | 129 | P_75@0 | S_54@0 | - | - |
| 75 | 75 |  | QPSK | 150 | P_75@0 | S_75@0 | - | - |
| 75 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 | - | - |
| 100 | 25 |  | QPSK | 125 | P_100@0 | S_25@0 | - | - |
| 100 | 25 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 100 | 50 |  | QPSK | 150 | P_100@0 | S_50@0 | - | - |
| 100 | 75 |  | QPSK | 175 | P_100@0 | S_75@0 | - | - |
| 100 | 100 |  | QPSK | 200 | P_100@0 | S_100@0 | - | - |
| 100 | 100 |  | QPSK | 130 | P_100@0 | S_30@0 | - | - |
| 100 | 100 |  | QPSK | 75 | P_75@0 | S_0@0 | - | - |
| 100 | 100 |  | QPSK | 100 | P_100@0 | S_0@0 | - | - |
| 100 | 100 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: Depending on CA configurations, only the appropriate Uplink RB allocation value according to table 7.3A.0-1 for UE supporting two uplink carriers is tested per Test CA configuration. Note 3: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. Note 4: For the DL signal one sided dynamic OCNG Pattern OP.1 FDD/TDD is used. |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The ULReference Measurement channel is set according to Table 6.5.1A.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.5.1A.1.4.3.

6.5.1A.1.4.2	Test procedure

1.	Configure SCC according to Annex C0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.5.1A.1.4.3

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.5.1A.1.4.1-1 on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Set the Downlink signal level for PCC and SCC to the appropriate REFSENS value defined in 7.3A.2.5. Send continuously uplink power control "up" commands in every uplink scheduling information to the UE to ensure the UE transmits PUMAX level for at least the duration of the test.

6.	Measure the Frequency Error on PCC and SCC using Global In-Channel Tx-Test (Annex E) respectively. For TDD slots with transient periods are not under test.

6.5.1A.1.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

##### 6.5.1A.1.5	Test Requirements

The 20 frequency error Δf results per test point must fulfil the test requirement:

|Δf| ≤ (0.1 PPM + 15 Hz) for each test point

where PPM refers to PCC UL frequency.

#### 6.5.1A.2	Frequency error for CA (inter-band DL CA and UL CA)

##### 6.5.1A.2.1	Test purpose

This test verifies the ability of both, receiver and the transmitter, to process frequency for inter-band CA correctly.

Receiver: to extract the correct frequency from the stimulus signal, offered by the System simulator, under ideal propagation conditions and low level.

Transmitter: to derive the correct modulated carrier frequency from the results, gained by the receiver.

##### 6.5.1A.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 11 and forward that support inter-band DL CA and UL CA.

##### 6.5.1A.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5.1A.0.

##### 6.5.1A.2.4	Test description

6.5.1A.2.4.1	Initial condition

Same as in clause 6.5.1A.1.4.1 with the following exceptions:

-	Instead of Table 5.4.2A.1-1 use Table 5.4.2A.1-2 for CA configurations with two bands or Table 5.4.2A.1-2a for CA configurations with three bands.

-	Instead of Table 6.5.1A.1.4.1-1  use Table 6.5.1A.2.4.1-1.

-	Instead of clause 6.5.1A.1.4.3 use clause 6.5.1A.2.4.3.

Table 6.5.1A.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  | Mid range for PCC and SCC |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |
| Network signalling value |  |  |  | NS_01 Unless given by Table 7.3.3-3 for the band with active uplink carrier |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |
| 6 | 25 | NA | QPSK | 31 | P_6@0 | S_25@0 |
| 6 | 50 |  | QPSK | 56 | P_6@0 | S_50@0 |
| 15 | 25 |  | QPSK | 40 | P_15@0 | S_25@0 |
| 25 | 6 |  | QPSK | 31 | S_25@0 | P_6@0 |
| 25 | 25 |  | QPSK | 50 | P_25@0 | S_25@0 |
| 25 | 75 |  | QPSK | 100 | S_25@0 | P_75@0 |
| 25 | 50 |  | QPSK | 75 | P_25@0 | S_50@0 |
| 50 | 50 |  | QPSK | 100 | P_50@0 | S_50@0 |
| 50 | 100 |  | QPSK | 150 | P_50@0 | S_100@0 |
| 75 | 25 |  | QPSK | 100 | P_75@0 | S_25@0 |
| 75 | 75 |  | QPSK | 150 | P_75@0 | S_75@0 |
| 75 | 100 |  | QPSK | 175 | P_75@0 | S_100@0 |
| 100 | 50 |  | QPSK | 100 | P_100@0 | S_50@0 |
| 100 | 100 |  | QPSK | 200 | P_100@0 | S_100@0 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: For the DL signal one sided dynamic OCNG Pattern OP.1 FDD/TDD is used. |  |  |  |  |  |  |

6.5.1A.2.4.2	Test procedure

Same test procedure as in clause 6.5.1A.1.4.2.

-	Instead of Table 6.5.1A.1.4.1-1  use Table 6.5.1A.2.4.1-1.

-	Instead of Table 7.3A.2.5-1 use Table 7.3A.6.5-1.

-	Instead of clause 6.5.1A.1.4.3 use clause6.5.1A.2.4.3.

6.5.1A.2.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

##### 6.5.1A.2.5	Test requirement

The 20 frequency error Δf results per test point must fulfil the test requirement:

|Δf| ≤ (0.1 PPM + 15 Hz) for each test point

where PPM refers to each CC UL frequency.

#### 6.5.1A.3	Frequency error for CA (intra-band non-contiguous DL CA and UL CA)

##### 6.5.1A.3.1	Test purpose

This test verifies the ability of both, receiver and the transmitter, to process frequency for intra-band non-contiguous CA correctly.

Receiver: to extract the correct frequency from the stimulus signal, offered by the System simulator, under ideal propagation conditions and low level.

Transmitter: to derive the correct modulated carrier frequency from the results, gained by the receiver.

##### 6.5.1A.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 11 and forward that support intra-band non-contiguous DL CA and UL CA.

##### 6.5.1A.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5.1A.0.

##### 6.5.1A.3.4	Test description

6.5.1A.3.4.1	Initial condition

Same as in clause 6.5.1A.1.4.1 with the following exceptions:

-	Instead of Table 5.4.2A.1-1 use Table 5.4.2A.1-3 for CA configurations.

-	Instead of Table 6.5.1A.1.4.1-1  use Table 6.5.1A.3.4.1-1.

-	Instead of clause 6.5.1A.1.4.3  use clause 6.5.1A.3.4.3.

Table 6.5.1A.3.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Refer to test points A: N/A (Range is not relevant), Maximum Wgap |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  | Refer to test point Test only test points with Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  | Wgap [MHz] | DL Allocation |  | UL Allocation |  |  |  |
|  | PCC NRB | SCCs NRB |  | PCC & SCC RB allocation |  | CC MOD | PCC NRB_alloc (LCRB @ RBstart) | CC MOD | SCC NRB_alloc (LCRB @ RBstart) |
| Test Parameters for CA_4A-4A Configurations |  |  |  |  |  |  |  |  |  |
| 1 | 25 | 25 | 35 | NA |  | QPSK | P_25@0 | QPSK | S_25@0 |
| 2 | 50 | 50 | 25 |  |  | QPSK | P_50@0 | QPSK | S_50@0 |
| 3 | 100 | 100 | 5 |  |  | QPSK | P_100@0 | QPSK | S_100@0 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-3. Note 2: Depending on CA configurations, only the appropriate Uplink RB allocation value according to table 7.3A.0-4 for UE supporting two uplink carriers is tested per Test CA configuration. Note 3: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. Note 4: For the DL signal one sided dynamic OCNG Pattern OP.1 FDD/TDD is used. |  |  |  |  |  |  |  |  |  |

6.5.1A.3.4.2	Test procedure

Same test procedure as in clause 6.5.1A.1.4.2 with the following exceptions:

-	Instead of Table 6.5.1A.1.4.1-1  use Table 6.5.1A.3.4.1-1.

-	Instead of Table 7.3A.2.5-1  7.3A.7.5-1.

-	Instead of clause 6.5.1A.1.4.3  use clause 6.5.1A.3.4.3.

6.5.1A.3.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

##### 6.5.1A.3.5	Test Requirements

The 20 frequency error Δf results per test point must fulfil the test requirement:

|Δf| ≤ (0.1 PPM + 15 Hz) for each test point

where PPM refers to each CC UL frequency.

#### 6.5.1A.4	Frequency error for CA (3UL CA)

Editor’s Note: This test case is incomplete. The following items are missing or are incomplete:

There is no intra-band non-contiguous combination for CA (3UL CA)

##### 6.5.1A.4.1	Test purpose

This test verifies the ability of both receiver and the transmitter to processes frequency for 3UL CA correctly.

Receiver: to extract the correct frequency from the stimulus signal, offered by the System simulator, under ideal propagation conditions and low level.

Transmitter: to derive the correct modulated carrier frequency from the results, gained by the receiver.

##### 6.5.1A.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 13 and forward that support intra-band contiguous or inter-band and 3UL CA.

##### 6.5.1A.4.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5.1A.0.

##### 6.5.1A.4.4	Test description

6.5.1A.4.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested inset up and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on E-UTRA CA configurations specified in table 5.4.2A.1-1. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 6.5.1A.4.4.1-1 to table 6.5.1A.4.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.1A.4.4.1-1: Intra Band Contiguous CA Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  | Low range, High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 3) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | UL Allocation |  |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation | CC MOD | NRB_alloc | PCC & SCC1 &SCC2 RB allocations (LCRB @ RBstart) |  |  |  |
| 25 | 50 | N/A | QPSK | 25 | P_25@0 | S_0@0 | S_0@0 | - |
| 25 | 50 |  | QPSK | 125 | P_25@0 | S_50@0 | S_50@0 | - |
| 50 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | S_0@0 | - |
| 50 | 50 |  | QPSK | 150 | P_50@0 | S_50@0 | S_50@0 | - |
| 75 | 75 |  | QPSK | 183 | P_75@0 | S_54@0 | S_54@0 | - |
| 75 | 75 |  | QPSK | 225 | P_75@0 | S_75@0 | S_75@0 | - |
| 75 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 | S_0@0 | - |
| 100 | 25 |  | QPSK | 150 | P_100@0 | S_25@0 | S_25@0 | - |
| 100 | 25 |  | QPSK | 50 | P_50@0 | S_0@0 | S_0@0 | - |
| 100 | 50 |  | QPSK | 200 | P_100@0 | S_50@0 | S_50@0 | - |
| 100 | 75 |  | QPSK | 250 | P_100@0 | S_75@0 | S_75@0 | - |
| 100 | 100 |  | QPSK | 300 | P_100@0 | S_100@0 | S_100@0 | - |
| 100 | 100 |  | QPSK | 160 | P_100@0 | S_30@0 | S_30@0 | - |
| 100 | 100 |  | QPSK | 75 | P_75@0 | S_0@0 | S_0@0 | - |
| 100 | 100 |  | QPSK | 100 | P_100@0 | S_0@0 | S_0@0 | - |
| 100 | 100 |  | QPSK | 50 | P_50@0 | S_0@0 | S_0@0 | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: Depending on CA configurations, only the appropriate Uplink RB allocation value according to table 7.3A.0-1 for UE supporting two uplink carriers is tested per Test CA configuration. Note 3: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. Note 4: For the DL signal one sided dynamic OCNG Pattern OP.1 FDD/TDD is used. |  |  |  |  |  |  |  |  |

Table 6.5.1A.4.4.1-2: Inter Band CA Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  | Mid range for PCC and SCC |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |
| Network signalling value |  |  |  | NS_01 Unless given by Table 7.3.3-3 for the band with active uplink carrier |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |
| 6 | 25 | NA | QPSK | 56 | P_6@0 | S_25@0 | S_25@0 |
| 6 | 50 |  | QPSK | 106 | P_6@0 | S_50@0 | S_50@0 |
| 15 | 25 |  | QPSK | 65 | P_15@0 | S_25@0 | S_25@0 |
| 25 | 25 |  | QPSK | 75 | P_25@0 | S_25@0 | S_25@0 |
| 25 | 50 |  | QPSK | 125 | P_25@0 | S_50@0 | S_50@0 |
| 50 | 50 |  | QPSK | 150 | P_50@0 | S_50@0 | S_50@0 |
| 50 | 100 |  | QPSK | 250 | P_50@0 | S_100@0 | S_100@0 |
| 75 | 75 |  | QPSK | 225 | P_75@0 | S_75@0 | S_75@0 |
| 75 | 100 |  | QPSK | 275 | P_75@0 | S_100@0 | S_100@0 |
| 100 | 50 |  | QPSK | 200 | P_100@0 | S_50@0 | S_50@0 |
| 100 | 100 |  | QPSK | 300 | P_100@0 | S_100@0 | S_100@0 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: For the DL signal one sided dynamic OCNG Pattern OP.1 FDD/TDD is used. |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure group A.55 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.5.1A.4.4.1-1 to 6.5.1A.4.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.5.1A.4.4.3.

6.5.1A.4.4.2	Test procedure

1.	Configure SCCs according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCCs as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.5.1A.4.4.3

3.	SS activates SCCs by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.5.1A.4.4.1-1 to 6.5.1A.4.4.1-2 on both PCC and SCCs. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Set the Downlink signal level for PCC and SCCs to the appropriate REFSENS value defined in 7.3A.2.5. Send continuously uplink power control "up" commands in every uplink scheduling information to the UE to ensure the UE transmits PUMAX level for at least the duration of the test.

6.	Measure the Frequency Error on PCC and SCCs using Global In-Channel Tx-Test (Annex E) respectively. For TDD slots with transient periods are not under test.

6.5.1A.4.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCCs configuration there are no additional message contents.

##### 6.5.1A.4.5	Test Requirements

The 20 frequency error Δf results per test point must fulfil the test requirement:

|Δf| ≤ (0.1 PPM + 15 Hz) for each test point

where PPM refers to PCC UL frequency.

#### 6.5.1A.5	Frequency error for CA (4UL CA)

##### 6.5.1A.5.1	Test purpose

This test verifies the ability of both receiver and the transmitter to processes frequency for 4UL CA correctly.

Receiver: to extract the correct frequency from the stimulus signal, offered by the System simulator, under ideal propagation conditions and low level.

Transmitter: to derive the correct modulated carrier frequency from the results, gained by the receiver.

##### 6.5.1A.5.2	Test applicability

This test case applies to all types of E-UTRA UE release 11 and forward that support 4DL CA and 4UL CA.

##### 6.5.1A.5.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5.1A.0.

##### 6.5.1A.5.4	Test description

6.5.1A.5.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested inset up and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on E-UTRA CA configurations specified in table 5.4.2A.1. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 6.5.1A.5.4.1-1 to 6.5.1A.5.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.1A.5.4.1-1: Intra Band Contiguous CA Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  | Low range, High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 3) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | UL Allocation |  |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCCs RB allocation | CC MOD | NRB_alloc | PCC & SCC1 & SCC2 & SCC3 RB allocations (LCRB @ RBstart) |  |  |  |
| 25 | 50 | N/A | QPSK | 25 | P_25@0 | S_0@0 | S_0@0 | S_0@0 |
| 25 | 50 |  | QPSK | 175 | P_25@0 | S_50@0 | S_50@0 | S_50@0 |
| 50 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | S_0@0 | S_0@0 |
| 50 | 50 |  | QPSK | 200 | P_50@0 | S_50@0 | S_50@0 | S_50@0 |
| 75 | 75 |  | QPSK | 237 | P_75@0 | S_54@0 | S_54@0 | S_54@0 |
| 75 | 75 |  | QPSK | 300 | P_75@0 | S_75@0 | S_75@0 | S_75@0 |
| 75 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 | S_0@0 | S_0@0 |
| 100 | 25 |  | QPSK | 175 | P_100@0 | S_25@0 | S_25@0 | S_25@0 |
| 100 | 25 |  | QPSK | 50 | P_50@0 | S_0@0 | S_0@0 | S_0@0 |
| 100 | 50 |  | QPSK | 250 | P_100@0 | S_50@0 | S_50@0 | S_50@0 |
| 100 | 75 |  | QPSK | 325 | P_100@0 | S_75@0 | S_75@0 | S_75@0 |
| 100 | 100 |  | QPSK | 400 | P_100@0 | S_100@0 | S_100@0 | S_100@0 |
| 100 | 100 |  | QPSK | 190 | P_100@0 | S_30@0 | S_30@0 | S_30@0 |
| 100 | 100 |  | QPSK | 75 | P_75@0 | S_0@0 | S_0@0 | S_0@0 |
| 100 | 100 |  | QPSK | 100 | P_100@0 | S_0@0 | S_0@0 | S_0@0 |
| 100 | 100 |  | QPSK | 50 | P_50@0 | S_0@0 | S_0@0 | S_0@0 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: Depending on CA configurations, only the appropriate Uplink RB allocation value according to table 7.3A.0-1 for UE supporting two uplink carriers is tested per Test CA configuration. Note 3: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. Note 4: For the DL signal one sided dynamic OCNG Pattern OP.1 FDD/TDD is used. |  |  |  |  |  |  |  |  |

Table 6.5.1A.5.4.1-2: Inter Band CA Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  | Mid range for PCC and SCC |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Network signalling value |  |  |  | NS_01 Unless given by Table 7.3.3-3 for the band with active uplink carrier |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | UL Allocation |  |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation | CC MOD | NRB_alloc | PCC & SCC1 & SCC2 & SCC3 RB allocations (LCRB @ RBstart) |  |  |  |
| 6 | 25 | N/A | QPSK | 81 | P_6@0 | S_25@0 | S_25@0 | S_25@0 |
| 6 | 50 |  | QPSK | 156 | P_6@0 | S_50@0 | S_50@0 | S_50@0 |
| 15 | 25 |  | QPSK | 90 | P_15@0 | S_25@0 | S_25@0 | S_25@0 |
| 25 | 25 |  | QPSK | 100 | P_25@0 | S_25@0 | S_25@0 | S_25@0 |
| 25 | 50 |  | QPSK | 175 | P_25@0 | S_50@0 | S_50@0 | S_50@0 |
| 50 | 50 |  | QPSK | 200 | P_50@0 | S_50@0 | S_50@0 | S_50@0 |
| 50 | 100 |  | QPSK | 350 | P_50@0 | S_100@0 | S_100@0 | S_100@0 |
| 75 | 75 |  | QPSK | 300 | P_75@0 | S_75@0 | S_75@0 | S_75@0 |
| 75 | 100 |  | QPSK | 375 | P_75@0 | S_100@0 | S_100@0 | S_100@0 |
| 100 | 50 |  | QPSK | 250 | P_100@0 | S_50@0 | S_50@0 | S_50@0 |
| 100 | 100 |  | QPSK | 400 | P_100@0 | S_100@0 | S_100@0 | S_100@0 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: For the DL signal one sided dynamic OCNG Pattern OP.1 FDD/TDD is used. |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure group A.55 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.5.1A.5.4.1-1 to 6.5.1A.5.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.5.1A.5.4.3.

6.5.1A.5.4.2	Test procedure

1.	Configure SCCs according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCCs as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.5.1A.5.4.3

3.	SS activates SCCs by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.5.1A.5.4.1-1 to 6.5.1A.5.4.1-2 on both PCC and SCCs. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Set the Downlink signal level for PCC and SCCs to the appropriate REFSENS value defined in 7.3A.2.5. Send continuously uplink power control "up" commands in every uplink scheduling information to the UE to ensure the UE transmits PUMAX level for at least the duration of the test.

6.	Measure the Frequency Error on PCC and SCCs using Global In-Channel Tx-Test (Annex E) respectively. For TDD slots with transient periods are not under test.

6.5.1A.5.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCCs configuration there are no additional message contents.

##### 6.5.1A.5	Test Requirements

The 20 frequency error Δf results per test point must fulfil the test requirement:

|Δf| ≤ (0.1 PPM + 15 Hz) for each test point

where PPM refers to PCC UL frequency.

### 6.5.1B	Frequency Error for UL-MIMO

#### 6.5.1B.1	Test purpose

This test verifies the ability of both, the receiver and the transmitter for UL-MIMO, to process frequency correctly.

Receiver: to extract the correct frequency from the stimulus signal, offered by the System simulator, under ideal propagation conditions and low level.

Transmitter: to derive the correct modulated carrier frequency for each transmitter connector from the results, gained by the receiver.

#### 6.5.1B.2	Test applicability

This test applies to all types of E-UTRA UE release 10 and forward that support UL-MIMO.

#### 6.5.1B.3	Minimum conformance requirements

The UE modulated carrier frequency for each transmit antenna shall be accurate to within ±0.1 PPM observed over a period of one time slot (0.5ms) compared to the carrier frequency received from the E-UTRA Node B.

The normative reference for this requirement is TS 36.101 clause 6.5.1B

#### 6.5.1B.4	Test description

##### 6.5.1B.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.5.1B.4.1-1. The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annexes A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.1B.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Lowest, 5MHz, Highest |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |  |
| Ch BW | Mod'n | RB allocation |  | Mod'n | RB allocation |  |
|  |  | FDD | TDD |  | FDD | TDD |
| 1.4MHz | QPSK | 6 | 6 | QPSK | 6 | 6 |
| 3MHz | QPSK | 15 | 15 | QPSK | 15 | 15 |
| 5MHz | QPSK | 25 | 25 | QPSK | 25 | 25 |
| 5MHz | QPSK | 25 | N/A | QPSK | 20 | N/A |
| 5MHz | QPSK | 25 | N/A | QPSK | 15 | N/A |
| 5MHz | QPSK | 25 | N/A | QPSK | 53 | N/A |
| 10MHz | QPSK | 50 | 50 | QPSK | 50 | 50 |
| 10MHz | QPSK | 50 | N/A | QPSK | 25 | N/A |
| 10MHz | QPSK | 50 | N/A | QPSK | 20 | N/A |
| 10MHz | QPSK | 50 | N/A | QPSK | 15 | N/A |
| 15MHz | QPSK | 75 | 75 | QPSK | 75 | 75 |
| 15MHz | QPSK | 75 | N/A | QPSK | 50 | N/A |
| 15MHz | QPSK | 75 | N/A | QPSK | 25 | N/A |
| 15MHz | QPSK | 75 | N/A | QPSK | 20 | N/A |
| 20MHz | QPSK | 100 | 100 | QPSK | 100 | 100 |
| 20MHz | QPSK | 100 | N/A | QPSK | 75 | N/A |
| 20MHz | QPSK | 100 | N/A | QPSK | 50 | N/A |
| 20MHz | QPSK | 100 | N/A | QPSK | 25 | N/A |
| 20MHz | QPSK | 100 | N/A | QPSK | 20 | N/A |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: Depending on E-UTRA band, only the appropriate Uplink RB allocation value according to table 7.3.3-2 is tested per Test Channel Bandwidth. Note 3: Applicable only to E-UTRA FDD Band 31. The UL resource blocks shall be located at RBstart 10 (according to Table 7.3.3-2). |  |  |  |  |  |  |

1.	Connect the SS to the UE to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.28.

2.	The parameter settings for the cell are set up according to TS 36.508[7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL and DL Reference Measurement channels are set according to Table 6.5.1B.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.5.1B.4.3.

##### 6.5.1B.4.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1A for C_RNTI to transmit the DL RMC according to Table 6.5.1B.4.1-1. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 4 for C_RNTI to schedule the UL RMC according to Table 6.5.1B.4.1-1, since the UE has no payload data to send, the UE transmit uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level to the appropriate REFSENS value defined in Table 7.3B.5-1. Send continuously uplink power control "up" commands to the UE in every uplink scheduling information to the UE so that the UE transmits at PUMAX level for the duration of the test.

4.	Measure the Frequency Error using Global In-Channel Tx-Test (Annex E) for each of transmit antenna of the UE. For TDD slots with transient periods are not under test.

##### 6.5.1B.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6 with the exceptions in clause 7.3.4.3 and Table 7.3.3-3.

#### 6.5.1B.5	Test requirement

The requirements apply to each transmit antenna connector.

The 20 frequency error Δf results must fulfil the test requirement:

|Δf| ≤ (0.1PPM+15 Hz)

### 6.5.1C	TBD

### 6.5.1D	Frequency Error for ProSe

#### 6.5.1D.0	Minimum conformance requirements

The UE modulated carrier frequency for ProSe sidelink transmissions shall be accurate to within ±0.1 PPM observed over a period of one time slot (0.5 ms) compared to the carrier frequency received from the synchronization source. The synchronization source can be E-UTRA Node B or a ProSe UE transmitting sidelink synchronization signals.

When UE is configured for simultaneous E-UTRA ProSe sidelink and E-UTRA uplink transmissions for inter-band E-UTRA ProSe / E-UTRA bands specified in Table 5.2D-2, the requirements in subclause 6.5.1D.0 apply for ProSe transmission and the requirements in subclause 6.5.1 apply for uplink transmission.

The normative reference for this requirement is TS 36.101 clause 6.5.1D.

#### 6.5.1D.1	Frequency error for ProSe Direct Discovery

Editor's note:	This test case is not complete. Following aspects are either missing or not yet determined:

-	ProSe UE authentication is not handled in the test case and is still under investigation.

-	Message exceptions content needs further investigation (some values still TBD).

-	Test procedure is not defined (concurrent operation with E-UTRA uplink transmissions and the communication between SS and UE are undefined).

##### 6.5.1D.1.1	Test purpose

This test verifies the ability of both, the receiver and the transmitter for ProSe Discovery physical channels, to process frequency correctly.

Receiver: to extract the correct frequency from the stimulus signal, offered by the System simulator, under ideal propagation conditions and low level.

Transmitter: to derive the correct modulated carrier frequency for each transmitter connector from the results, gained by the receiver.

##### 6.5.1D.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 12 and forward which support ProSe Direct Discovery, but not ProSe Direct Communication.

##### 6.5.1D.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5.1D.0.

##### 6.5.1D.1.4	Test description

6.5.1D.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA ProSe bands specified in table 5.4.2D.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.5.1D.1.4.1-1. The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annexes A.2, A.3 and A.6.

Table 6.5.1D.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 5MHz, Highest |  |  |  |
|  | Downlink Configuration |  | Uplink Configuration |  | Sidelink Configuration (PSDCH) |  |
| Ch BW | Mod’n | RB allocation | Mod’n | RB allocation | Mod’n | RB allocation |
| 5MHz | QPSK | 8 | QPSK | 8 | QPSK | 2 |
| 10MHz | QPSK | 16 | QPSK | 16 | QPSK | 2 |
| 15MHz | QPSK | 25 | QPSK | 25 | QPSK | 2 |
| 20MHz | QPSK | 30 | QPSK | 30 | QPSK | 2 |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.67.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] clause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.1.

4.	The UL and DL Reference Measurement channels are set according to Table 6.5.1D.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 2A-RF according to TS 36.508 [7] clause 5.2A.1A. Message contents exceptions are defined in clause 6.5.1D.1.4.3.

7. 	SS continues with the generic procedures described in TS 36.508 [7] clause 4.5.3A.3 and 4.5.4.3 and ensures the UE is in State 4 according to TS 36.508 [7] clause 4.5.4 and the UE test loop Mode D is closed and UE is set to announce discovery.

6.5.1D.1.4.2	Test procedure

1.	SS shall transmit an RRCConnectionReconfiguration message to configure DRX. Message contents exceptions (only those marked with “…Step 1…” ) are defined in clause 6.5.1D.1.4.3.

2.	The UE shall transmit RRCConnectionReconfigurationComplete message.

3.	SS transmits PDSCH via PDCCH DCI format 1A for C_RNTI to transmit the DL RMC according to Table 6.5.1D.1.4.1-1. The SS sends downlink MAC padding bits on the DL RMC.

4.	SS sends uplink scheduling information via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.5.1D.1.4.1-1, since the UE has no payload data to send, the UE transmit uplink MAC padding bits on the UL RMC.

5.	SS transmits PDSCH to transmit the SL RMC according to Table 6.5.1D.1.4.1-1. Set the Sidelink signal level to the appropriate REFSENS value defined in Table 7.3D.1.5-1.

6.	Measure the Frequency Error using Global In-Channel Tx-Test (Annex E) for each of transmit antenna of the UE. For TDD slots with transient periods are not under test.

6.5.1D.1.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6.

Message contents are according to TS 36.508 [7] clauses TBD with following exceptions:

-	Table A.7.1.1-1 for FDD

-	Table A.7.1.2-1 for TDD

Table 6.5.1D.1.4.3-1: ACTIVATE TEST MODE: Frequency Error for ProSe Direct Discovery

Derivation Path: 36.508, Table 4.7A-1, condition UE TEST LOOP MODE D

Table 6.5.1D.1.4.3-2: CLOSE UE TEST LOOP: Frequency Error for ProSe Direct Discovery

Derivation Path: 36.508, Table 4.7A-3, condition UE TEST LOOP MODE D

Table 6.5.1D.1.4.3-3: MAC-MainConfig-RBC: Frequency Error for ProSe Direct Discovery (applicable only for Step 1 of Test procedure)

| Derivation Path: TS 36.508 [7] clause 4.8.2.1.5, Table 4.8.2.1.5-1 MAC-MainConfig-RBC |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MAC-MainConfig-RBC ::= SEQUENCE { |  |  |  |  |
| ul-SCH-Config SEQUENCE { |  |  |  |  |
| maxHARQ-Tx |  | TBD |  |  |
| periodicBSR-Timer |  | TBD |  |  |
| retxBSR-Timer |  | sf2560 |  |  |
| ttiBundling |  | FALSE |  |  |
| } |  |  |  |  |
| drx-Config CHOICE { |  |  |  |  |
| Setup SEQUENCE { |  |  |  |  |
| onDurationTimer |  | psf1 |  |  |
| drx-InactivityTimer |  | psf1 |  |  |
| drx-RetransmissionTimer |  | psf1 |  |  |
| longDRX-CycleStartOffset CHOICE { |  |  |  |  |
| sf2560 |  | 0 |  |  |
| } |  |  |  |  |
| shortDRX |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| timeAlignmentTimerDedicated |  | TBD |  |  |
| } |  |  |  |  |

##### 6.5.1D.1.5	Test requirement

The 20 frequency error Δf results must fulfil the test requirement:

|Δf| ≤ (0.1 PPM + 15 Hz)

#### 6.5.1D.2	Frequency error for ProSe Direct Communication

Editor's note:	This test case is not complete. Following aspects are either missing or not yet determined:

-	Many clauses are TBD.

##### 6.5.1D.2.1	Test purpose

This test verifies the ability of both, the receiver and the transmitter for ProSe Communication physical channels, to process frequency correctly.

Receiver: to extract the correct frequency from the stimulus signal, offered by the System simulator, under ideal propagation conditions and low level.

Transmitter: to derive the correct modulated carrier frequency for each transmitter connector from the results, gained by the receiver.

##### 6.5.1D.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 12 and forward which support ProSe Direct Communication.

##### 6.5.1D.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5.1D.0.

##### 6.5.1D.2.4	Test description

TBD

6.5.1D.2.4.1	Initial condition

TBD

6.5.1D.2.4.2	Test procedure

TBD

6.5.1D.2.4.3	Message contents

TBD

##### 6.5.1D.2.5	Test requirements

The 20 frequency error Δf results must fulfil the test requirement:

|Δf| ≤ (0.1 PPM + 15 Hz)

### 6.5.1E	Frequency Error for UE category 0

#### 6.5.1E.1	Test purpose

Same test purpose as in clause 6.5.1.1.

#### 6.5.1E.2	Test applicability

This test case applies to all types of E-UTRA UE release 12 and forward of UE category 0.

#### 6.5.1E.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.1.3.

#### 6.5.1E.4	Test description

##### 6.5.1E.4.1	Initial condition

Same initial conditions as in clause 6.5.1.4.1 with following exceptions:

-	Instead of Table 6.5.1.4.1-1  use Table 6.5.1E.4.1-1.

-	Connect SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.3 using only main UE Tx/Rx antenna.

Table 6.5.1E.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Lowest, 5MHz, Highest |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |  |
| Ch BW | Mod'n | RB allocation |  | Mod'n | RB allocation |  |
|  |  | FDD and HD-FDD | TDD |  | FDD and HD-FDD | TDD |
| 1.4MHz | QPSK | 6 | 6 | QPSK | 6 | 6 |
| 3MHz | QPSK | 14 | 14 | QPSK | 15 | 15 |
| 5MHz | QPSK | 14 | 14 | QPSK | 25 | 25 |
| 5MHz | QPSK | 14 | N/A | QPSK | 20 | N/A |
| 5MHz | QPSK | 14 | N/A | QPSK | 15 | N/A |
| 10MHz | QPSK | 14 | 14 | QPSK | 50 | 36 |
| 10MHz | QPSK | 14 | N/A | QPSK | 25 | N/A |
| 10MHz | QPSK | 14 | N/A | QPSK | 20 | N/A |
| 10MHz | QPSK | 14 | N/A | QPSK | 15 | N/A |
| 15MHz | QPSK | 14 | 14 | QPSK | 36 | 36 |
| 15MHz | QPSK | 14 | N/A | QPSK | 36 | N/A |
| 15MHz | QPSK | 14 | N/A | QPSK | 25 | N/A |
| 15MHz | QPSK | 14 | N/A | QPSK | 20 | N/A |
| 20MHz | QPSK | 14 | 14 | QPSK | 36 | 36 |
| 20MHz | QPSK | 14 | N/A | QPSK | 36 | N/A |
| 20MHz | QPSK | 14 | N/A | QPSK | 25 | N/A |
| 20MHz | QPSK | 14 | N/A | QPSK | 20 | N/A |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: Depending on E-UTRA band, only the appropriate Uplink RB allocation value according to table 7.3.3-2 is tested per Test Channel Bandwidth. |  |  |  |  |  |  |

##### 6.5.1E.4.2	Test procedure

Same test procedure as in clause 6.5.1.4.2 with following exception for HD-FDD:

-	In step 4, slots with transient periods are not under test. Half-duplex guard sub frame are not under test.

##### 6.5.1E.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the exceptions in subclause 7.3.4.3 and Table 7.3.3-3.

#### 6.5.1E.5	Test requirement

The 20 frequency error Δf results must fulfil the test requirement:

|Δf| ≤ (0.1 PPM + 15 Hz)

### 6.5.1EA	Frequency Error for UE category M1

#### 6.5.1EA.1	Test purpose

Same test purpose as in clause 6.5.1.1.

#### 6.5.1EA.2	Test applicability

This test case applies to all types of E-UTRA UE release 13 and forward of UE category M1.

#### 6.5.1EA.3	Minimum conformance requirements

For category M1 TDD UEs and FD-FDD UEs, the frequency error requirements in Clause 6.5.1.3 apply.

For category M1 HD-FDD UEs and for continuous uplink transmissions of duration ≤ 64 ms, the frequency error requirements in Clause 6.5.1.3 apply.

For category M1 HD-FDD UEs and for continuous uplink transmissions of duration > 64 ms, the UE modulated carrier frequency shall be accurate to within the limits in Table 6.5.1EA.3-1 observed over a period of one time slot (0.5 ms) compared to the carrier frequency received from the E-UTRA Node B.

Table 6.5.1EA.3-1: Frequency error requirement for HD-FDD UE category M1

| Carrier frequency [GHz] | Frequency error [ppm] |
| --- | --- |
| ≤1 | ±0.2 |
| >1 | ±0.1 |

The normative reference for this requirement is TS 36.101 clause 6.5.1E

#### 6.5.1EA.4	Test description

##### 6.5.1EA.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in sub-clause 5.2E. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.5.1EA.4.1-1. The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annexes A.2 and A.3. Configurations of PDSCH and MPDCCH before measurement are specified in Annex C.2.

Table 6.5.1EA.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 5MHz |  |  |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |  |  |
| Ch BW | Modulation | RB allocation |  | Modulation | RB allocation |  |  |
|  |  | FDD and HD-FDD | TDD |  | FDD and HD-FDD | TDD | Narrowband index1 |
| 5MHz | QPSK | 4 | 4 | QPSK | 6 | 6 | 0 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4 |  |  |  |  |  |  |  |

1.	Connect the SS to the UE to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508[7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL and DL Reference Measurement channels are set according to Table 6.5.1EA.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [7] clause 5.2A.2AA. Message contents are defined in clause 6.5.1EA.4.3.

##### 6.5.1EA.4.2	Test procedure

1.	SS transmits PDSCH via MPDCCH DCI format 6-1A for C_RNTI to transmit the DL RMC according to Table 6.5.1EA.4.1-1. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via MPDCCH DCI format 6-0A for C_RNTI to schedule the UL RMC according to Table 6.5.1EA.4.1-1, since the UE has no payload data to send, the UE transmit uplink MAC padding bits on the UL RMC

3.	Set the Downlink signal level to the appropriate REFSENS value defined in Table 7.3EA-1. Send continuously uplink power control "up" commands to the UE in every uplink scheduling information to the UE so that the UE transmits at PUMAX level for the duration of the test.

4.	Measure the Frequency Error using Global In-Channel Tx-Test (Annex E) according to the UE’s declaration on the position of carrier centre frequency. For TDD and HDD-FD slots with transient periods are not under test. Half-duplex guard sub frame are not under test

##### 6.5.1EA.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the condition CEModeA and the exceptions in subclause 7.3EA.4.3.

#### 6.5.1EA.5	Test requirement

The 20 frequency error Δf results must fulfil the test requirement:

|Δf| ≤ (0.1 PPM + 15 Hz)

### 6.5.1EA_1	Frequency Error for UE category M1 (CEmodeB)

#### 6.5.1EA_1.1	Test purpose

Same test purpose as in clause 6.5.1.1.

#### 6.5.1EA_1.2	Test applicability

This test case applies to all types of E-UTRA HD-FDD UE release 13 and forward of UE category M1 that support operating in CEmodeB.

#### 6.5.1EA_1.3	Minimum conformance requirements

For category M1 TDD UEs and FD-FDD UEs, the frequency error requirements in Clause 6.5.1.3 apply.

For category M1 HD-FDD UEs and for continuous uplink transmissions of duration ≤ 64 ms, the frequency error requirements in Clause 6.5.1.3 apply.

For category M1 HD-FDD UEs and for continuous uplink transmissions of duration > 64 ms, the UE modulated carrier frequency shall be accurate to within the limits in Table 6.5.1EA_1.3-1 observed over a period of one time slot (0.5 ms) compared to the carrier frequency received from the E-UTRA Node B.

Table 6.5.1EA_1.3-1: Frequency error requirement for HD-FDD UE category M1

| Carrier frequency [GHz] | Frequency error [ppm] |
| --- | --- |
| ≤1 | ±0.2 |
| >1 | ±0.1 |

The normative reference for this requirement is TS 36.101 clause 6.5.1E

#### 6.5.1EA_1.4	Test description

##### 6.5.1EA_1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in sub-clause 5.2E. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.5.1EA_1.4.1-1. The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annexes A.2 and A.3. Configurations of PDSCH and MPDCCH before measurement are specified in Annex C.2.

Table 6.5.1EA_1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 5MHz |  |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |  |
| Ch BW | Modulation | RB allocation |  | Modulation | RB allocation |  |
|  |  | HD-FDD |  |  | HD-FDD | Narrowband index1 |
| 5MHz | QPSK | 4 |  | QPSK | 2 | 0 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4 |  |  |  |  |  |  |

1.	Connect the SS to the UE to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508[7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL and DL Reference Measurement channels are set according to Table 6.5.1EA_1.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [7] clause 5.2A.2AA. Message contents are defined in clause 6.5.1EA_1.4.3.

##### 6.5.1EA_1.4.2	Test procedure

1.	SS transmits PDSCH via MPDCCH DCI format 6-1B for C_RNTI to transmit the DL RMC according to Table 6.5.1EA_1.4.1-1. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information via MPDCCH DCI format 6-0B for C_RNTI to schedule the UL RMC according to Table 6.5.1EA_1.4.1-1, since the UE has no payload data to send, the UE transmit uplink MAC padding bits on the UL RMC

3.	Set the Downlink signal level to the appropriate REFSENS value defined in Table 7.3EA-1. Send continuously uplink power control "up" commands to the UE in every uplink scheduling information to the UE so that the UE transmits at PUMAX level for the duration of the test.

4.	Measure the Frequency Error using Global In-Channel Tx-Test (Annex E). The acquired frame for Frequency Error measurement starts at 2020ms after the start of the first scheduled PUSCH subframe (Note 1).

Note 1: 2020ms is the start of the last UL frame before the last UL compensation gap within the PUSCH repetition period of 2048ms. The same frame for measuring of Frequency Error applies irrespective of the UL compensation gap existence. UL compensation gaps exist if the UE capability, ue-CE-NeedULGaps-r13, is true. See 36.211 [8], clause 5.3.4 for UL compensating gap timing for CEmodeB UEs.

##### 6.5.1EA_1.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the condition CEModeB and the exceptions in subclause 7.3EA.4.3, Table 7.3EA.3-3 and:

Table 6.5.1EA_1.4.3-1: PUSCH-ConfigCommon-v1310-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-10A PUSCH-ConfigCommon-v1310-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigCommon-v1310-DEFAULT ::= SEQUENCE { |  |  |  |
| pusch-maxNumRepetitionCEmodeB-r13 | r2048 | Set to maximum value for CE Mode B | CEmodeB |
| } |  |  |  |

#### 6.5.1EA_1.5	Test requirement

The 20 frequency error Δf results must fulfil the test requirement:

| Carrier frequency [GHz] | Frequency error ¦Δf¦ [Hz] |
| --- | --- |
| ≤1 | ≤ (0.2ppm + 15 Hz) |
| >1 | ≤ (0.1ppm + 15 Hz) |

### 6.5.1EB	Frequency Error for UE category 1bis

#### 6.5.1EB.1	Test purpose

Same test purpose as in clause 6.5.1.

#### 6.5.1EB.2	Test applicability

This test applies to all types of E-UTRA UE release 13 and forward of UE category 1bis.

#### 6.5.1EB.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.1.3.

#### 6.5.1EB.4	Test description

##### 6.5.1EB.4.1	Initial conditions

Same initial conditions as in clause 6.5.1.4.1 with the following exception:

-	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.3 using only main UE Tx/Rx antenna.

##### 6.5.1EB.4.2	Test procedure

Same test procedure as in clause 6.5.1.4.2 with following exceptions:

3.	Set the Downlink signal level to the appropriate REFSENS value defined in Table 7.3EB.5-1. Send continuously uplink power control "up" commands to the UE in every uplink scheduling information to the UE so that the UE transmits at PUMAX level for the duration of the test.

##### 6.5.1EB.4.3	Message contents

Same message contents as in clause 6.5.1.4.3.

#### 6.5.1EB.5	Test requirement

The 20 frequency error Δf results must fulfil the test requirement:

|Δf| ≤ (0.1 PPM + 15 Hz)

### 6.5.1EC	Frequency Error for UE category M2

#### 6.5.1EC.1	Test purpose

Same as in clause 6.5.1EA.1

#### 6.5.1EC.2	Test applicability

This test case applies to all types of E-UTRA UE release 14 and forward of UE category M2.

#### 6.5.1EC.3	Minimum conformance requirements

Same as in clause 6.5.1EA.3

#### 6.5.1EC.4	Test description

##### 6.5.1EC.4.1	Initial condition

Same as in clause 6.5.1EA.4.1 with the following exceptions:

-	Instead of Table 6.5.1EA.4.1-1  use Table 6.5.1EC.4.1-1

-	Instead of Clause 6.5.1EA.4.3 use Clause 6.5.1EC.4.3

Table 6.5.1EC.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 5MHz |  |  |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |  |  |
| Ch BW | Modulation | RB allocation |  | Modulation | RB allocation |  |  |
|  |  | FDD and HD-FDD | TDD |  | FDD and HD-FDD | TDD | Narrowband index1 |
| 5MHz | QPSK | 16 | 16 | QPSK | 24 | 24 | 0 |
| Note 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.4. |  |  |  |  |  |  |  |

##### 6.5.1EC.4.2	Test procedure

Same as in clause 6.5.1EA.4.1 with the following exceptions:

-	Instead of Table 6.5.1EA.4.1-1 use Table 6.5.1EC.4.1-1

-	Instead of Table 7.3EA-1 use Table 7.3EC.3-1

##### 6.5.1EC.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the condition CEModeA and the exceptions in subclause 7.3EC.4.3.

#### 6.5.1EC.5	Test requirements

Same as in clause 6.5.1EA.5.

### 6.5.1EC_1	Frequency Error for UE category M2 (CE Mode B)

#### 6.5.1EC_1.1	Test purpose

Same test purpose as in clause 6.5.1.1.

#### 6.5.1EC_1.2	Test applicability

This test case applies to all types of E-UTRA HD-FDD UE release 14 and forward of UE category M2 that support operating in CEmodeB.

#### 6.5.1EC_1.3	Minimum conformance requirements

For category M1 and M2 TDD UEs and FD-FDD UEs, the frequency error requirements in Clause 6.5.1.3 apply.

For category M1 and M2 HD-FDD UEs and for continuous uplink transmissions of duration ≤ 64 ms, the frequency error requirements in Clause 6.5.1.3 apply.

For category M1 and M2 HD-FDD UEs and for continuous uplink transmissions of duration > 64 ms, the UE modulated carrier frequency shall be accurate to within the limits in Table 6.5.1EC_1.3-1 observed over a period of one time slot (0.5 ms) compared to the carrier frequency received from the E-UTRA Node B.

Table 6.5.1EC_1.3-1: Frequency error requirement for HD-FDD UE category M1 and M2

| Carrier frequency [GHz] | Frequency error [ppm] |
| --- | --- |
| ≤1 | ±0.2 |
| >1 | ±0.1 |

The normative reference for this requirement is TS 36.101 clause 6.5.1E

#### 6.5.1EC_1.4	Test description

##### 6.5.1EC_1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in sub-clause 5.2E. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.5.1EC_1.4.1-1. The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annexes A.2 and A.3. Configurations of PDSCH and MPDCCH before measurement are specified in Annex C.2.

Table 6.5.1EC_1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Highest |  |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |  |
| Ch BW | Modulation | RB allocation |  | Modulation | RB allocation |  |
|  |  | HD-FDD |  |  | HD-FDD | Narrowband index1 |
| 5MHz | QPSK | 4 |  | QPSK | 6 | 0 |
| 10MHz | QPSK | 4 |  | QPSK | 6 | 0 |
| 15MHz | QPSK | 4 |  | QPSK | 6 | 0 |
| 20MHz | QPSK | 4 |  | QPSK | 6 | 0 |
| Note 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.4. |  |  |  |  |  |  |

1.	Connect the SS to the UE to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508[7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL and DL Reference Measurement channels are set according to Table 6.5.1EC_1.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [7] clause 5.2A.2AA. Message contents are defined in clause 6.5.1EC_1.4.3.

##### 6.5.1EC_1.4.2	Test procedure

1.	SS transmits PDSCH via MPDCCH DCI format 6-1B for C_RNTI to transmit the DL RMC according to Table 6.5.1EC_1.4.1-1. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information via MPDCCH DCI format 6-0B for C_RNTI to schedule the UL RMC according to Table 6.5.1EC_1.4.1-1, since the UE has no payload data to send, the UE transmit uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level to the value defined in Table 7.3EC.3-2. Send continuously uplink power control "up" commands to the UE in every uplink scheduling information to the UE so that the UE transmits at PUMAX level for the duration of the test.

4.	Measure the Frequency Error using Global In-Channel Tx-Test (Annex E). The acquired frame for Frequency Error measurement starts at 2020ms after the start of the first scheduled PUSCH subframe (Note 1).

Note 1:	2020ms is the start of the last UL frame before the last UL compensation gap within the PUSCH repetition period of 2048ms. The same frame for measuring of Frequency Error applies irrespective of the UL compensation gap existence. UL compensation gaps exist if the UE capability, ue-CE-NeedULGaps-r13, is true. See 36.211 [8], clause 5.3.4 for UL compensating gap timing for CEmodeB UEs.

##### 6.5.1EC_1.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the condition CEModeB and the exceptions in subclause 7.3EA.4.3 and:

Table 6.5.1EC_1.4.3-1: PUSCH-ConfigCommon-v1310-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-10A PUSCH-ConfigCommon-v1310-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigCommon-v1310-DEFAULT ::= SEQUENCE { |  |  |  |
| pusch-maxNumRepetitionCEmodeB-r13 | r2048 | Set to maximum value for CE Mode B | CEmodeB |
| } |  |  |  |

#### 6.5.1EC_1.5	Test requirement

The 20 frequency error Δf results must fulfil the test requirement:

| Carrier frequency [GHz] | Frequency error ¦Δf¦ [Hz] |
| --- | --- |
| ≤1 | ≤ (0.2ppm + 15 Hz) |
| >1 | ≤ (0.1ppm + 15 Hz) |

### 6.5.1F	Frequency Error for category NB1 and NB2

6.5.1F.1	Test purpose

This test verifies the ability of both, the receiver and the transmitter, to process frequency correctly.

Receiver: to extract the correct frequency from the stimulus signal, offered by the System simulator, under ideal propagation conditions and low level.

Transmitter: to derive the correct modulated carrier frequency from the results, gained by the receiver.

6.5.1F.2	Test applicability

This test case applies to all types of NB-IoT FDD UE release 13 and forward of category NB1.

This test case applies to all types of NB-IoT FDD UE release 14 and forward of category NB2.

This test case applies to all types of NB-IoT TDD UE release 15 and forward of category NB1 and NB2.

6.5.1F.3	Minimum conformance requirements

For UE category NB1 and NB2, the UE modulated carrier frequency shall be accurate to within the following limits

Table 6.5.1F.3-1: Frequency error requirement for UE category NB1 and NB2

| Carrier frequency [GHz] | Frequency error [ppm] |
| --- | --- |
| ≤1 | ±0.2 |
| >1 | ±0.1 |

Observed over a period of one time slot (0.5 ms for 15 kHz sub-carrier spacing and 2 ms excluding the 2304Ts gap for 3.75 kHz sub-carrier spacing) and averaged over 72/LCtones slots (where LCtones = {1, 3, 6, 12} is the number of sub-carriers used for the transmission), compared to the carrier frequency received from the E-UTRA Node B.

The normative reference for this requirement is TS 36.101 clause 6.5.1F.

#### 6.5.1F.4	Test description

##### 6.5.1F.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions and test frequencies based on the subset of E-UTRA bands defined for NB-IOT in clause 5.2F. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.5.1F.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2.4. Configurations of NPDSCH and NPDCCH before measurement are specified in Annex C.2.

Table 6.5.1F.4.1-1: Test Configuration Table for FDD & TDD

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 8.1.1 |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 8.1.3.1 |  | Frequency ranges defined in Annex K.1.1 |  |  |
| Test Parameters |  |  |  |  |
| Configuration ID | Downlink Configuration | Uplink Configuration |  |  |
|  |  | Modulation | Ntones | Subcarrier spacing |
| 1 |  | QPSK | 1@0 | 3.75 |
| 2 |  | QPSK | 1@0 | 15 |
| 3 (Note 1) |  | QPSK | 3@0 | 15 |
| 4 (Note 1) |  | QPSK | 6@0 | 15 |
| 5 (Note 1) |  | QPSK | 12@0 | 15 |
| Note 1: Applicable to UE supporting UL multi-tone transmissions |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A, in Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex [H.1 and H.3.0].

4.	The UL Reference Measurement channel is set according to Table 6.5.1F.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5. Message contents are defined in clause 6.5.1F.4.3.

##### 6.5.1F.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via NPDCCH DCI format N0 for C_RNTI to schedule the UL RMC according to Table 6.5.1F.4.1-1, since the UE has no payload data to send, the UE transmit uplink MAC padding bits on the UL RMC. (UE should be already transmitting PUMAX after Initial Conditions setting).

2.	Set the Downlink signal level to the appropriate REFSENS value defined in Table 7.3F.1.5-1. For the DL signal, Narrowband IoT OCNG pattern 1 in Annex A.5.3.1 is used.

3.	Measure the Frequency Error using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test.

NOTE 1:	For configuration IDs applicable to UE depending on UE capability in Test Configuration Table with different UL sub-carrier spacing, the SS shall release the connection through State 3A-NB and finally ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5 using the appropriate UL subcarrier spacing in Random Access Response message.

##### 6.5.1F.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 8.1.6.

#### 6.5.1F.5	Test requirement

The 20 frequency error Δf results must fulfil the test requirement:

|Δf| ≤ (0.1 PPM + 15 Hz) (Carrier frequency >1 GHz)

|Δf| ≤ (0.2 PPM + 15 Hz) (Carrier frequency ≤1 GHz)

### 6.5.1G	Frequency Error for V2X Communication

#### 6.5.1G.1	Frequency Error for V2X Communication / Non-concurrent with E-UTRA uplink transmission

##### 6.5.1G.1.1	Test purpose

This test verifies the ability of both, the receiver and the transmitter, to process frequency correctly.

Receiver: to extract the correct frequency from the stimulus signal, offered by the System simulator, under ideal propagation conditions and low level.

Transmitter: to derive the correct modulated carrier frequency from the results, gained by the receiver.

##### 6.5.1G.1.2	Test applicability

This test case applies to all types of UE that support V2X Sidelink communication and Band 47.

##### 6.5.1G.1.3	Minimum Conformance Requirements

The UE modulated carrier frequency for V2X sidelink transmissions shall be accurate to within ±0.1 PPM observed over a period of one time slot (0.5 ms) compared to the absolute frequency in case of using GNSS synchronization source. The same requirements applied over a period of one time slot (0.5 ms) compared to the relative frequency in case of using the E-UTRA Node B or V2X UE sidelink synchronization signals.

When UE is configured for simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions for inter-band E-UTRA V2X / E-UTRA bands specified in Table 5.2G-1, the requirements in TS 36.101[2] subclause 6.5.1G apply for V2X sidelink transmission and the requirements in TS 36.101[2] subclause 6.5.1 apply for the E-UTRA uplink transmission.

The normative reference for this requirement is TS 36.101[2] clause 6.5.1G

##### 6.5.1G.1.4	Test description

###### 6.5.1G.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in table 5.4.2G.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.5.1G.1.4.1-1. The details of the V2X reference measurement channels (RMCs) are specified in Annex A.8.3 and the GNSS configuration in TS 36.508 [7] subclause 4.11.

Table 6.5.1G.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  | Mid range |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |
|  | V2X Configuration to Transmit |  |  |
| Ch BW | Modulation |  | PSSCH RB allocation |
| 10MHz | QPSK |  | 48@2 |
| 20MHz | QPSK |  | 96@2 |

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.89.

2.	The parameter settings for the V2X sidelink transmission over PC5 for both SS and UE are pre-configured according to TS 36.508 [7] subclause 4.10.1. Message content exceptions are defined in clause 6.5.1G.1.4.3.

3.	The V2X reference measurement channel is set according to Table 6.5.1G.1.4.1-1.

4.	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS36.508 [7] Table 4.11.2-2. Geographical area #1 is also pre-configured in the UE.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 5A-V2X in Transmit Mode according to TS 36.508 [7] clause 4.5.9.

7.	The GNSS simulator is triggered to start step 1 of Scenario #1 to simulate a location in the centre of Geographical area #1. Wait for the UE to acquire the GNSS signal and start to transmit.

###### 6.5.1G.1.4.2	Test procedure

1.	The UE starts to perform the V2X sidelink communication according to SL-V2X-Preconfiguration. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the V2X RMC.

2.	The UE transmits at PUMAX level according to pre-configured parameters for the duration of the test.

3.	Measure the Frequency Error using Global In-Channel Tx-Test (Annex E).

###### 6.5.1G.1.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.7.I and 4.10.

##### 6.5.1G.1.5	Test requirement

The 20 frequency error Δf results must fulfil the test requirement:

|Δf| ≤ (0.1 PPM + 36 Hz)

#### 6.5.1G.2	Frequency error for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions

6.5.1G.2.1	Test Purpose

This test verifies the ability of both, the receiver and the transmitter, to process frequency correctly.

Receiver: to extract the correct frequency from the stimulus signal, offered by the System simulator, under ideal propagation conditions and low level.

Transmitter: to derive the correct modulated carrier frequency from the results, gained by the receiver.

6.5.1G.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 14 and forward that support simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions for inter-band E-UTRA V2X / E-UTRA bands specified in TS 36.101 [2] Table 5.2G -2.

6.5.1G.2.3	Minimum conformance requirements

The UE modulated carrier frequency for V2X sidelink transmissions shall be accurate to within ±0.1 PPM observed over a period of one time slot (0.5 ms) compared to the absolute frequency in case of using GNSS synchronization source. The same requirements applied over a period of one time slot (0.5 ms) compared to the relative frequency in case of using the E-UTRA Node B or V2X UE sidelink synchronization signals.

When UE is configured for simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions for inter-band E-UTRA V2X / E-UTRA bands specified in TS 36.101 [2] Table 5. 2G -2, the requirements in subclause 6.5.1G.1.3 apply for V2X sidelink transmission and the requirements in subclause 6.5.1.3 apply for the E-UTRA uplink transmission.

The normative reference for this requirement is TS 36.101 [2] clause 6.5.1G.

6.5.1G.2.4	Test description

6.5.1G.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on Inter-band con-current V2X configurations specified in Table 5.4.2G.1-2. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.5.1G.2.4.1-1. The details of the uplink and V2X reference measurement channels (RMCs) are specified in Annexes A.2 and A.8.3 respectively. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.1G.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  |  |  |  | Low range, Mid range, High range |  |  |  |  |
| Test Channel Bandwidths as specified in 5.4.2G |  |  |  |  |  | Lowest NRB_agg, Highest NRB_agg |  |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |  |  |  |  |
|  | Downlink |  | Uplink Configuration |  |  | V2X Configuration |  |  |  |  |
| Ch BW | Mod'n | RB allocation | Mod'n |  | RB allocation | Ch BW | Mod'n | PSSCH RB allocation |  |  |
| 1.4 MHz | QPSK | 6 | QPSK |  | 6 | 10 MHz | QPSK | 48@2 |  |  |
| 3 MHz | QPSK | 15 | QPSK |  | 15 | 20 MHz | QPSK | 96@2 |  |  |
| 5 MHz | QPSK | 25 | QPSK |  | 25 |  |  |  |  |  |
| 10 MHz | QPSK | 50 | QPSK |  | 50 |  |  |  |  |  |
| 15 MHz | QPSK | 75 | QPSK |  | 75 |  |  |  |  |  |
| 20 MHz | QPSK | 100 | QPSK |  | 100 |  |  |  |  |  |
| Note 1: V2X con-current band configurations are checked separately for each band configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2G.1-2. |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.93a.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL and V2X reference measurement channels are set according to Table 6.5.1G.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF-V2X according to TS 36.508 [7] clause 5.2A.2C. Message content exceptions are defined in clause 6.5.1G.2.4.3.

6.5.1G.2.4.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1A for C_RNTI to transmit the DL RMC according to Table 6.5.1G.2.4.1-1. The SS sends downlink MAC padding bits on the DL RMC.

2.	The SS simultaneously sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC and V2X scheduling information via PDCCH DCI format 5A for SL_V_RNTI to schedule the V2X RMC according to Table 6.5.1G.2.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL and V2X RMCs.

3.	Set the Downlink signal level to the appropriate REFSENS value defined in Table 7.3.5-1. Send continuously uplink power control "up" commands in the uplink scheduling information to the UE to ensure that the UE transmits at PUMAX level for the duration of the test. Configure the UE to transmit PSCCH and PSSCH with the PUMAX level of each test points.

4.	Measure the Frequency Error over all component carriers in the V2X con-current band configuration using Global In-Channel Tx-Test (Annex E). On E-UTRA carrier, the TDD slots with transient periods are not under test.

6.5.1G.2.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the exceptions in subclause 7.3.4.3 and Table 7.3.3-3.

6.5.1G.2.5	Test requirement

The 20 frequency error Δf results for both concurrent bands must fulfil the test requirement:

|Δf| ≤ (0.1 PPM + 15 Hz), f ≤ 3.0GHz

|Δf| ≤ (0.1 PPM + 36 Hz), 4.2GHz <f ≤ 6.0GHz

### 6.5.2	Transmit modulation

Transmit modulation defines the modulation quality for expected in-channel RF transmissions from the UE. This transmit modulation limit is specified in terms of:

-	Error Vector Magnitude (EVM) for the allocated resources blocks (RB),

-	EVM equalizer spectrum flatness derived from the equalizer coefficients generated by the EVM measurement process

-	Carrier leakage

In-band emissions for the non-allocated RB

#### 6.5.2.1	Error Vector Magnitude (EVM)

##### 6.5.2.1.1	Test Purpose

The Error Vector Magnitude is a measure of the difference between the reference waveform and the measured waveform. This difference is called the error vector. Before calculating the EVM the measured waveform is corrected by the sample timing offset and RF frequency offset. Then the carrier leakage shall be removed from the measured waveform before calculating the EVM.

The measured waveform is further modified by selecting the absolute phase and absolute amplitude of the Tx chain. The EVM result is defined after the front-end IDFT as the square root of the ratio of the mean error vector power to the mean reference power expressed as a %.

The basic EVM measurement interval in the time domain is one preamble sequence for the PRACH and is one slot for the PUCCH and PUSCH in the time domain. When the PUSCH or PUCCH transmission slot is shortened due to multiplexing with SRS, the EVM measurement interval is reduced by one symbol, accordingly. Likewise, when the PUSCH starting position is modified or when second last symbol is the ending symbol of the PUSCH sub-frame for Frame Structure Type 3, the EVM measurement interval is reduced accordingly. The PUSCH or PUCCH EVM measurement interval is also reduced when the mean power, modulation or allocation between slots is expected to change. In the case of PUSCH transmission, the measurement interval is reduced by a time interval equal to the sum of 5 μs and the applicable exclusion period defined in subclause 6.3.4, adjacent to the boundary where the power change is expected to occur. The PUSCH exclusion period is applied to the signal obtained after the front-end IDFT. In the case of PUCCH transmission, the PUCCH EVM measurement interval is reduced by one symbol adjacent to the slot boundary.

##### 6.5.2.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 8 and forward.

##### 6.5.2.1.3	Minimum conformance requirements

EVM measurements are evaluated for 10 uplink sub-frames excluding any transient period for the average EVM case, and 60 subframes excluding any transient period for the reference signal EVM case, the different modulations schemes shall not exceed the values specified in Table 6.5.2.1.3-1 for the parameters defined in Table 6.5.2.1.3-2. For EVM evaluation purposes, [all PRACH preamble formats 0-4 and] all PUCCH formats 1, 1a, 1b, 2, 2a and 2b are considered to have the same EVM requirement as QPSK modulated.

Table 6.5.2.1.3-1: Minimum requirements for Error Vector Magnitude

| Parameter | Unit | Average EVM Level | Reference Signal EVM Level |
| --- | --- | --- | --- |
| QPSK or BPSK | % | 17.5 | 17.5 |
| 16QAM | % | 12.5 | 12.5 |

Table 6.5.2.1.3-2: Parameters for Error Vector Magnitude

| Parameter | Unit | Level |
| --- | --- | --- |
| UE Output Power | dBm | -40 |
| Operating conditions |  | Normal conditions |

The normative reference for this requirement is TS 36.101 [2] clause 6.5.2.1.1.

##### 6.5.2.1.4	Test description

6.5.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.5.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.2.1.4.1-1: Test Configuration Table for PUSCH

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  | NC |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  | See Table 6.5.1.4.1-1 |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  | See Table 6.5.1.4.1-1 |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
|  | Downlink Configuration | Uplink Configuration |  |  |
| Ch BW | N/A for PUSCH EVM testing | Modulation | RB allocation |  |
|  |  |  | FDD | TDD |
| 1.4MHz |  | QPSK | 6 | 6 |
| 1.4MHz |  | QPSK | 1 | 1 |
| 1.4MHz |  | 16QAM | 6 | 6 |
| 1.4MHz |  | 16QAM | 1 | 1 |
| 3MHz |  | QPSK | 15 | 15 |
| 3MHz |  | QPSK | 4 | 4 |
| 3MHz |  | 16QAM | 15 | 15 |
| 3MHz |  | 16QAM | 4 | 4 |
| 5MHz |  | QPSK | 25 | 25 |
| 5MHz |  | QPSK | 8 | 8 |
| 5MHz |  | 16QAM | 25 | 25 |
| 5MHz |  | 16QAM | 8 | 8 |
| 10MHz |  | QPSK | 50 | 50 |
| 10MHz |  | QPSK | 12 | 12 |
| 10MHz |  | 16QAM | 50 (Note 3) | 50 (Note 3) |
| 10MHz |  | 16QAM | 12 | 12 |
| 15MHz |  | QPSK | 75 | 75 |
| 15MHz |  | QPSK | 16 | 16 |
| 15MHz |  | 16QAM | 75 (Note 3) | 75 (Note 3) |
| 15MHz |  | 16QAM | 16 | 16 |
| 20MHz |  | QPSK | 100 | 100 |
| 20MHz |  | QPSK | 18 | 18 |
| 20MHz |  | 16QAM | 100 (Note 3) | 100 (Note 3) |
| 20MHz |  | 16QAM | 18 | 18 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: For partial RB allocation, the RBstart shall be RB #0 and RB# (max+1 - RB allocation) of the channel bandwidth. Note 3: Applies only for UE-Categories ≥2 |  |  |  |  |

Table 6.5.2.1.4.1-2: Test Configuration Table for PUCCH

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  | NC |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 |  |  | See Table 6.5.1.4.1-1 |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | See Table 6.5.1.4.1-1 |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |
| Ch BW | Mod'n | RB allocation |  | FDD: PUCCH format = Format 1a TDD: PUCCH format = Format 1a / 1b |
|  |  | FDD | TDD |  |
| 1.4MHz | QPSK | 3 | 3 |  |
| 3MHz | QPSK | 4 | 4 |  |
| 5MHz | QPSK | 8 | 8 |  |
| 10MHz | QPSK | 16 | 16 |  |
| 15MHz | QPSK | 25 | 25 |  |
| 20MHz | QPSK | 30 | 30 |  |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. |  |  |  |  |

Table 6.5.2.1.4.1-3: Test Configuration for PRACH

| Initial Conditions |  |  |
| --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) | NC |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) | See Table 6.5.1.4.1-1 |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) | See Table 6.5.1.4.1-1 |  |
| PRACH preamble format |  |  |
|  | FDD | TDD |
| PRACH Configuration Index | 4 | 53 |
| RS EPRE setting for test point 1 (dBm/15kHz) | -71 | -63 |
| RS EPRE setting for test point 2 (dBm/15kHz) | -86 | -78 |

1.	Connect the SS to the UE to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.3.

2.	The parameter settings for the cell are set up according to TS 36.508[7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to in Table 6.5.2.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.5.2.1.4.3.

6.5.2.1.4.2	Test procedure

Test procedure for PUSCH:

1.1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.5.2.1.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

1.2	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

1.3	Measure the EVM and  using Global In-Channel Tx-Test (Annex E).

1.4	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is –36.8dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -36.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

1.5	Measure the EVM and  using Global In-Channel Tx-Test (Annex E).

Test procedure for PUCCH:

2.1	PUCCH are set according to Table 6.5.2.1.4.1-2.

2.2	SS transmits PDSCH via PDCCH DCI format 1A for C_RNTI to transmit the DL RMC according to Table 6.5.2.1.4.1-2. The SS sends downlink MAC padding bits on the DL RMC. The transmission of PDSCH will make the UE send uplink ACK/NACK using PUCCH. There is no PUSCH transmission.

2.3	SS send appropriate TPC commands for PUCCH to the UE until the UE transmit PUCCH at PUMAX level.

2.4	Measure PUCCH EVM using Global In-Channel Tx-Test (Annex E).

2.5	Send the appropriate TPC commands for PUCCH to the UE until the UE transmits PUCCH at -36.8dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -36.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

2.6	Measure PUCCH EVM using Global In-Channel Tx-Test (Annex E).

Test procedure for PRACH:

3.1	The SS shall set RS EPRE according to Table 6.5.2.1.4.1-3.

3.2	PRACH is set according to Table 6.5.2.1.4.1-3.

3.3	The SS shall signal a Random Access Preamble ID via a PDCCH order to the UE and initiate a Non-contention based Random Access procedure

3.4	The UE shall send the signalled preamble to the SS.

3.5	In response to the preamble, the SS shall transmit a random access response not corresponding to the transmitted random access preamble, or send no response.

3.6	The UE shall consider the random access response reception not successful then re-transmit the preamble with the calculated PRACH transmission power.

3.7	Repeat step 5 and 6 until the SS collect enough PRACH preambles (2 preambles for format 0 and 10 preambles for format 4). Measure the EVM in PRACH channel using Global In-Channel Tx-Test (Annex E).

6.5.2.1.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the following exceptions:

Table 6.5.2.1.4.3-1: PRACH-ConfCommonDEFAULT: PRACH EVM measurement for FDD

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-7 PRACH-ConfCommonDEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PRACH-ConfigInfo SEQUENCE { |  |  |  |
| prach-ConfigIndex | 4 |  |  |

Table 6.5.2.1.4.3-2: PRACH-ConfCommonDEFAULT: PRACH EVM measurement for TDD

| Derivation Path: TS 36.508 [7] clause 5.3.1, Table 5.3.1-1 PRACH-ConfCommonDEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PRACH-ConfigInfo SEQUENCE { |  |  |  |
| prach-ConfigIndex | 53 |  |  |

Table 6.5.2.1.4.3-4: RACH-ConfigCommon-DEFAULT: PRACH EVM measurement

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-12 RACH-ConfigCommon-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon-DEFAULT ::= SEQUENCE { |  |  |  |
| preambleInfo SEQUENCE { |  |  |  |
| numberOfRA-Preambles | n52 |  |  |
| preamblesGroupAConfig SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| powerRampingParameters SEQUENCE { |  |  |  |
| powerRampingStep | dB0 |  |  |
| preambleInitialReceivedTargetPower | dBm-120 Test point 1 dBm-90 Test point 2 |  |  |
| } |  |  |  |
| ra-SupervisionInfo SEQUENCE { |  |  |  |
| preambleTransMax | n10 |  | FDD |
|  | n20 |  | TDD |
| ra-ResponseWindowSize | Sf10 |  |  |
| mac-ContentionResolutionTimer | sf48 |  |  |
| } |  |  |  |
| ra-SupervisionInfo SEQUENCE { |  |  |  |

Table 6.5.2.1.4.3-5: TDD-Config-DEFAULT: PRACH EVM measurement for TDD

| Derivation Path: TS 36.508 [7] clause 5.3.1, Table 5.3.1-1: TDD-Config-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-Config-DEFAULT ::= SEQUENCE { |  |  |  |
| subframeAssignment | sa1 |  |  |
| specialSubframePatterns | ssp5 | To enable two symbol UpPTS, and to have 9 symbols GP. |  |
| } |  |  |  |

##### 6.5.2.1.5	Test requirement

The PUSCH EVM derived in E.4.2 shall not exceed 17,5 % for QPSK and BPSK, 12,5% for 16 QAM.

The PUSCH derived in E.4.6.2 shall not exceed 17,5 % when embedded with data symbols of QPSK and BPSK, 12,5% for 16 QAM.

The PUCCH EVM and derived in E.5.9.2 shall not exceed 17,5 %.

The PRACH EVM derived in FFS shall not exceed 17.5%.

#### 6.5.2.1_1	Error Vector Magnitude (EVM) for UL 64QAM

##### 6.5.2.1_1.1	Test Purpose

Same test purpose as in clause 6.5.2.1.1

##### 6.5.2.1_1.2	Test applicability

This test applies to E-UTRA UE from release 13 and forward that support UL 64QAM.

##### 6.5.2.1_1.3	Minimum conformance requirements

The RMS average of the basic EVM measurements for 10 sub-frames excluding any transient period for the average EVM case, and 60 sub-frames excluding any transient period for the reference signal EVM case, for 64QAM shall not exceed the value specified in Table 6.5.2.1_1.3-1 for the parameters defined in Table 6.5.2.1_1.3-2.

Table 6.5.2.1_1.3-1: Minimum requirements for Error Vector Magnitude

| Parameter | Unit | Average EVM Level | Reference Signal EVM Level |
| --- | --- | --- | --- |
| 64QAM | % | 8 | 8 |

Table 6.5.2.1_1.3-2: Parameters for Error Vector Magnitude

| Parameter | Unit | Level |
| --- | --- | --- |
| UE Output Power | dBm | -40 |
| Operating conditions |  | Normal conditions |

The normative reference for this requirement is TS 36.101 [2] clause 6.5.2.1.1.

##### 6.5.2.1_1.4	Test description

6.5.2.1_1.4.1		Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.5.2.1_1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.2.1_1.4.1-1: Test Configuration Table for PUSCH

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  | NC |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  | See Table 6.5.1.4.1-1 |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  | See Table 6.5.1.4.1-1 |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
|  | Downlink Configuration | Uplink Configuration |  |  |
| Ch BW | N/A for PUSCH EVM testing | Mod'n | RB allocation |  |
|  |  |  | FDD | TDD |
| 1.4MHz |  | 64QAM | 6 | 6 |
| 1.4MHz |  | 64QAM | 1 | 1 |
| 3MHz |  | 64QAM | 15 | 15 |
| 3MHz |  | 64QAM | 4 | 4 |
| 5MHz |  | 64QAM | 25 | 25 |
| 5MHz |  | 64QAM | 8 | 8 |
| 10MHz |  | 64QAM | 50 | 50 |
| 10MHz |  | 64QAM | 12 | 12 |
| 15MHz |  | 64QAM | 75 | 75 |
| 15MHz |  | 64QAM | 16 | 16 |
| 20MHz |  | 64QAM | 100 | 100 |
| 20MHz |  | 64QAM | 18 | 18 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: For partial RB allocation, the RBstart shall be RB #0 and RB# (max+1 - RB allocation) of the channel bandwidth. |  |  |  |  |

1.	Connect the SS to the UE to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.3.

2.	The parameter settings for the cell are set up according to TS 36.508[7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to in Table 6.5.2.1_1.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.5.2.1_1.4.3.

6.5.2.1_1.4.2	Test procedure

Same test procedure as in clause 6.5.2.1.4.2

6.5.2.1_1.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6

##### 6.5.2.1_1.5	Test requirement

The PUSCH EVM derived in E.4.2 shall not exceed 8% for 64QAM.

The PUSCH derived in E.4.6.2 shall not exceed 8% for 64QAM.

#### 6.5.2.1_2	Error Vector Magnitude (EVM) for UL 256QAM

##### 6.5.2.1_2.1	Test Purpose

Same test purpose as in clause 6.5.2.1.1

##### 6.5.2.1_2.2	Test applicability

This test case applies to all types of E-UTRA UE release 14 and forward that support UL 256QAM.

##### 6.5.2.1_2.3	Minimum conformance requirements

The RMS average of the basic EVM measurements for 10 sub-frames excluding any transient period for the average EVM case, and 60 sub-frames excluding any transient period for the reference signal EVM case, for 256QAM shall not exceed the value specified in Table 6.5.2.1_2.3-1 for the parameters defined in Table 6.5.2.1_2.3-2.

Table 6.5.2.1_2.3-1: Minimum requirements for Error Vector Magnitude

| Parameter | Unit | Average EVM Level | Reference Signal EVM Level |
| --- | --- | --- | --- |
| 256QAM | % | 3.5 | 3.5 |

Table 6.5.2.1_2.3-2: Parameters for Error Vector Magnitude

| Parameter | Unit | Level |
| --- | --- | --- |
| UE Output Power for 256QAM | dBm | -30 |
| Operating conditions |  | Normal conditions |

The normative reference for this requirement is TS 36.101 [2] clause 6.5.2.1.1.

##### 6.5.2.1_2.4	Test description

6.5.2.1_2.4.1		Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.5.2.1_2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.2.1_2.4.1-1: Test Configuration Table for PUSCH

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  | NC |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  | Lowest, 5MHz, Highest |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
|  | Downlink Configuration | Uplink Configuration |  |  |
| Ch BW | N/A for PUSCH EVM testing | Mod'n | RB allocation |  |
|  |  |  | FDD | TDD |
| 1.4MHz |  | 256QAM | 6 | 6 |
| 3MHz |  | 256QAM | 15 | 15 |
| 5MHz |  | 256QAM | 25 | 25 |
| 10MHz |  | 256QAM | 50 | 50 |
| 15MHz |  | 256QAM | 75 | 75 |
| 20MHz |  | 256QAM | 100 | 100 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in Table 5.4.2.1-1. |  |  |  |  |

1.	Connect the SS to the UE to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.3.

2.	The parameter settings for the cell are set up according to TS 36.508[7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to in Table 6.5.2.1_2.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.5.2.1_2.4.3.

6.5.2.1_2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.5.2.1.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the EVM and  using Global In-Channel Tx-Test (Annex E).

4.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is –26.8dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -26.5dBm ±3.5dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

5.	Measure the EVM and  using Global In-Channel Tx-Test (Annex E).

6.5.2.1_2.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6

##### 6.5.2.1_2.5	Test requirement

The PUSCH EVM derived in Annex E.4.2 shall not exceed the values in Table 6.5.2.1_2.5-1 for 256QAM.

The PUSCH derived in Annex E.4.6.2 shall not exceed the values in Table 6.5.2.1_2.5-1 when embedded with data symbols of 256QAM.

Table 6.5.2.1_2.5-1: Test requirements for 256QAM Error Vector Magnitude

| UL Power | Unit | Average EVM Level |
| --- | --- | --- |
| 15 dBm < PUL | % | 3.8 |
| -25 dBm < PUL≤ 15 dBm | % | 4.3 |
| -30dBm ≤ PUL ≤ -25dBm | % | 4.6 |

#### 6.5.2.1A	PUSCH-EVM with exclusion period

##### 6.5.2.1A.1	Test purpose

To verify the ability of the UE transmitter to keep the EVM minimum requirements, even in the presence of transients according to subclause 6.5.2.1.1 third paragraph:

In the case of PUSCH transmission, the measurement interval is reduced by a time interval equal to the sum of 5 μs and the applicable exclusion period defined in subclause 6.3.4, adjacent to the boundary where the power change is expected to occur. The PUSCH exclusion period is applied to the signal obtained after the front-end IDFT.

##### 6.5.2.1A.2	Test applicability

This test applies to all types of E-UTRA UE release 8 and forward.

##### 6.5.2.1A.3	Minimum conformance requirement

EVM measurements are evaluated for 10 uplink sub-frames in a reduced time interval due to exclusion periods for the average EVM. The different modulations schemes shall not exceed the values specified in Table 6.5.2.1.3-1 for the parameters defined in Table 6.5.2.1.3-2.

##### 6.5.2.1A.4	Test description

6.5.2.1A.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.5.2.1A.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.2.1A.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  | Normal Conditions |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 |  | Low range |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  | 10 MHz |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
|  | Downlink Configuration |  | Uplink Configuration |  |  |
| Ch BW | N/A |  | Mod'n | RB allocation |  |
|  |  |  |  | FDD | TDD |
| 10MHz |  |  | QPSK | Alternating 12 and 1 (as shown in Figure 6.5.2.1A.4.2-1) | Alternating 12 and 1 (as shown in Figure 6.5.2.1A.4.2-1) |
| 10MHz |  |  | 16 QAM | Alternating 12 and 1 (as shown in Figure 6.5.2.1A.4.2-1) | Alternating 12 and 1 (as shown in Figure 6.5.2.1A.4.2-1) |

1. Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.3.

2. The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3. Downlink signals are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4. The UL Reference Measurement channel is set according to table 6.5.2.1A.4.1-1

5. Propagation conditions are set according to Annex B.0.

6. Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.3.5.2.4.3.

6.5.2.1A.4.2	Test procedure

The test pattern is illustrated in figure 6.5.2.1A.4.2-1.

Figure 6.5.2.1A.4.2-1: Test pattern

NOTE 1:	In TDD the free subframes are special subframes or DL, in FDD the free subframes are OFF.

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the PUSCH... Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC. The initial uplink RB allocation is 12. Send appropriate TPC commands for PUSCH to the UE to ensure the UE transmits PUSCH at 0dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or 0dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

2.	Schedule the UE's PUSCH data transmission as described in Figure 6.5.2.1A.4.2-1 for 161) active time slots with an uplink RB allocation alternating pattern as described in table 6.5.2.1A.4.1-1 while transmitting 0dB TPC command for PUSCH via the PDCCH.

3.	Measure the EVM using Global In-Channel Tx-Test. The averaging across 161) timeslots is done across mixed RB allocations, as illustrated in Figure 6.5.2.1A.4.2-1

NOTE 2: Averaging across 16 timeslots is used to represent each type of transition equally in the average.

##### 6.5.2.1A.5	Test requirement

The PUSCH EVM derived in Annex E.4.2 taking into account Annex E.7 shall not exceed 17,5 % for QPSK and 12,5% for 16 QAM. The test requirements shall be fulfilled for early and late EVM window.

#### 6.5.2.1E	Error Vector Magnitude (EVM) for UE category 0

##### 6.5.2.1E.1	Error Vector Magnitude (EVM) for UE category 0

6.5.2.1E.1.1	Test Purpose

Same test purpose as in clause 6.5.2.1.1.

6.5.2.1E.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 12 and forward of UE category 0.

6.5.2.1E.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.2.1.3.

6.5.2.1E.1.4	Test description

6.5.2.1E.1.4.1		Initial conditions

Same initial conditions as in clause 6.5.2.1.4.1 with following exceptions:

-	Instead of Table 6.5.2.1.4.1-1 to Table 6.5.2.1.4.1-3  use Table 6.5.2.1E.4.1-1 to Table 6.5.2.1E.4.1-3.

-	Connect SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.3 using only main UE Tx/Rx antenna.

Table 6.5.2.1E.1.4.1-1: Test Configuration Table for PUSCH

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  | NC |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  | See Table 6.5.1.4.1-1 |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  | See Table 6.5.1.4.1-1 |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
|  | Downlink Configuration | Uplink Configuration |  |  |
| Ch BW | N/A for PUSCH EVM testing | Mod'n | RB allocation |  |
|  |  |  | FDD and HD-FDD | TDD |
| 1.4MHz |  | QPSK | 6 | 6 |
| 1.4MHz |  | QPSK | 1 | 1 |
| 1.4MHz |  | 16QAM | 5 | 5 |
| 1.4MHz |  | 16QAM | 1 | 1 |
| 3MHz |  | QPSK | 15 | 15 |
| 3MHz |  | QPSK | 4 | 4 |
| 3MHz |  | 16QAM | 5 | 5 |
| 3MHz |  | 16QAM | 4 | 4 |
| 5MHz |  | QPSK | 25 | 25 |
| 5MHz |  | QPSK | 8 | 8 |
| 5MHz |  | 16QAM | 5 | 5 |
| 10MHz |  | QPSK | 36 | 36 |
| 10MHz |  | QPSK | 12 | 12 |
| 10MHz |  | 16QAM | 5 | 5 |
| 15MHz |  | QPSK | 36 | 36 |
| 15MHz |  | QPSK | 16 | 16 |
| 15MHz |  | 16QAM | 5 | 5 |
| 20MHz |  | QPSK | 36 | 36 |
| 20MHz |  | QPSK | 18 | 18 |
| 20MHz |  | 16QAM | 5 | 5 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: For partial RB allocation, the RBstart shall be RB #0 and RB# (max+1 - RB allocation) of the channel bandwidth. |  |  |  |  |

Table 6.5.2.1E.1.4.1-2: Test Configuration Table for PUCCH

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  | NC |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 |  |  | See Table 6.5.1.4.1-1 |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | See Table 6.5.1.4.1-1 |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |
| Ch BW | Mod'n | RB allocation |  | FDD and HD-FDD: PUCCH format = Format 1a TDD: PUCCH format = Format 1a / 1b |
|  |  | FDD | TDD |  |
| 1.4MHz | QPSK | 3 | 3 |  |
| 3MHz | QPSK | 4 | 4 |  |
| 5MHz | QPSK | 8 | 8 |  |
| 10MHz | QPSK | 14 | 14 |  |
| 15MHz | QPSK | 14 | 14 |  |
| 20MHz | QPSK | 14 | 14 |  |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. |  |  |  |  |

Table 6.5.2.1E.1.4.1-3: Test Configuration for PRACH

| Initial Conditions |  |  |
| --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) | NC |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) | See Table 6.5.1.4.1-1 |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) | See Table 6.5.1.4.1-1 |  |
| PRACH preamble format |  |  |
|  | FDD and HD-FDD | TDD |
| PRACH Configuration Index | 4 | 53 |
| RS EPRE setting for test point 1 (dBm/15kHz) | -71 | -63 |
| RS EPRE setting for test point 2 (dBm/15kHz) | -86 | -78 |

6.5.2.1E.1.4.2	Test procedure

Same test procedure as in clause 6.5.2.1E.1.4.2.

6.5.2.1E.1.4.3	Message contents

Same message contents as in clause 6.5.2.1E.1.4.3.

6.5.2.1E.1.5	Test requirement

The PUSCH EVM derived in E.4.2 shall not exceed 17,5 % for QPSK and BPSK, 12,5% for 16 QAM.

The PUSCH derived in E.4.6.2 shall not exceed [17,5 %] when embedded with data symbols of QPSK and BPSK, [12,5%] for 16 QAM.

The PUCCH EVM and derived in E.5.9.2 shall not exceed 17,5 %.

The PRACH EVM derived in FFS shall not exceed 17.5%.

##### 6.5.2.1E.2	PUSCH-EVM with exclusion period for UE category 0

6.5.2.1E.2.1	Test purpose

Same test purpose as in clause 6.5.2.1A.1

6.5.2.1E.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 12 and forward of UE category 0.

6.5.2.1E.2.3	Minimum conformance requirement

Same minimum conformance requirements as in clause 6.5.2.1A.3

6.5.2.1E.2.4	Test description

6.5.2.1E.2.4.1	Initial conditions

Same initial conditions as in clause 6.5.2.1.2.4.1 with following exceptions for HD-FDD:

-	Instead of Table 6.5.2.1.2.4.1-1  use Table 6.5.2.1E.2.4.1-1.

-	Connect SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.3 using only main UE Tx/Rx antenna.

Table 6.5.2.1E.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  | Normal Conditions |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 |  | Low range |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  | 10 MHz |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
|  | Downlink Configuration |  | Uplink Configuration |  |  |
| Ch BW | N/A |  | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | TDD |
| 10MHz |  |  | QPSK | Alternating 12 and 1 (as shown in Figure 6.5.2.1E.2.4.2-1) | Alternating 12 and 1 (as shown in Figure 6.5.2.1E.2.4.2-1) |
| 10MHz |  |  | 16 QAM | Alternating 5 and 1 (as shown in Figure 6.5.2.1E.2.4.2-1) | Alternating 5 and 1 (as shown in Figure 6.5.2.1E.2.4.2-1) |

6.5.2.1E.2.4.2	Test procedure

The test pattern is illustrated in figure 6.5.2.1E.2.4.2-1.

Figure 6.5.2.1E.2.4.2-1: Test pattern

NOTE 1:	In TDD the free subframes are special subframes or DL, in FDD the free subframes are OFF.

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the PUSCH... Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC. The initial uplink RB allocation is 12 or 5. Send appropriate TPC commands for PUSCH to the UE to ensure the UE transmits PUSCH at 0dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or 0dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

2.	Schedule the UE's PUSCH data transmission as described in Figure 6.5.2.1E.2.4.2-1 for 161) active time slots with an uplink RB allocation alternating pattern as described in table 6.5.2.1E.2.4.1-1 while transmitting 0dB TPC command for PUSCH via the PDCCH.

3.	Measure the EVM using Global In-Channel Tx-Test. The averaging across 161) timeslots is done across mixed RB allocations, as illustrated in Figure 6.5.2.1E.2.4.2-1

NOTE 2:	Averaging across 16 timeslots is used to represent each type of transition equally in the average.

6.5.2.1E.2.5	Test requirement

The PUSCH EVM derived in Annex E.4.2 taking into account Annex E.7 shall not exceed 17,5 % for QPSK and 12,5% for 16 QAM. The test requirements shall be fulfilled for early and late EVM window.

#### 6.5.2.1EA	Error Vector Magnitude (EVM) for UE category M1

##### 6.5.2.1EA.1	Error Vector Magnitude (EVM) for UE category M1

6.5.2.1EA.1.1	Test Purpose

Same test purpose as in clause 6.5.2.1.1.

6.5.2.1EA.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 13 and forward of UE category M1.

6.5.2.1EA.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.2.1.3.

6.5.2.1EA.1.4	Test description

6.5.2.1EA.1.4.1		Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands 	. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.5.2.1EA.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and MPDCCH before measurement are specified in Annex C.2.

Table 6.5.2.1EA.1.4.1-1: Test Configuration Table for PUSCH

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  | NC |  |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  | Low range, Mid range, High range |  |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  | 5MHz |  |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  | Downlink Configuration | Uplink Configuration |  |  |  |
| Ch BW | N/A for PUSCH EVM testing | Mod'n | RB allocation |  |  |
|  |  |  | FDD and HD-FDD | TDD | Narrowband index1 |
| 5MHz |  | QPSK | 6 | 6 | 0 |
| 5MHz |  | QPSK | 1 (Note 3), 3 (Note 4) | 1 (Note 3), 3 (Note 4) | 0 |
| 5MHz |  | 16QAM | 5 | 5 | 0 |
| 5MHz |  | 16QAM | 1 (Note 3), 3 (Note 4) | 1 (Note 3), 3 (Note 4) | 0 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4 Note 2: For partial RB allocation, the RBstart shall be RB #0 and RB# (6 - RB allocation) of the channel bandwidth. Note 3: Only applicable for Power class 3 Note 4: Only applicable for Power class 5 and Power class 6 |  |  |  |  |  |

Table 6.5.2.1EA.1.4.1-2: Test Configuration Table for PUCCH

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  | NC |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 5MHz |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
|  | Downlink Configuration |  |  |  | Uplink Configuration |
| Ch BW | Mod'n | RB allocation |  |  | FDD and HD-FDD: PUCCH format = Format 1a TDD: PUCCH format = Format 1a / 1b |
|  |  | FDD | TDD | Narrowband index2 |  |
| 5MHz | QPSK | 4 | 4 | 0 |  |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4. Note 3: The RBstart of partial RB allocation can be either RB#0 or RB# (6 - RB allocation) of the narrowband. |  |  |  |  |  |

Table 6.5.2.1EA.1.4.1-3: Test Configuration for PRACH

| Initial Conditions |  |  |
| --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) | NC |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) | Low range, Mid range, High range |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) | 5MHz, 15MHz |  |
| PRACH preamble format |  |  |
|  | FDD and HD-FDD | TDD |
| PRACH Configuration Index | 4 | 53 |
| RS EPRE setting for test point 1 (dBm/15kHz) | -71 | -63 |
| RS EPRE setting for test point 2 (dBm/15kHz) | -86 | -78 |

1.	Connect the SS to the UE to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508[7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to in Table 6.5.2.1EA.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [7] clause 5.2A.2AA. Message contents are defined in clause 6.5.2.1EA.4.3.

6.5.2.1EA.1.4.2	Test procedure

Test procedure for PUSCH:

1.1.	SS sends uplink scheduling information for each UL HARQ process via MPDCCH DCI format 6-0A for C_RNTI to schedule the UL RMC according to Table 6.5.2.1EA.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

1.2	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

1.3	Measure the EVM and  using Global In-Channel Tx-Test (Annex E) according to the UE’s declaration on the position of carrier centre frequency.

1.4	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is –36.8dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -36.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

1.5	Measure the EVM and  using Global In-Channel Tx-Test (Annex E) according to the UE’s declaration on the position of carrier centre frequency.

Test procedure for PUCCH:

2.1	PUCCH are set according to Table 6.5.2.1EA.4.1-2.

2.2	SS transmits PDSCH via MPDCCH DCI format 6-1A for C_RNTI to transmit the DL RMC according to Table 6.5.2.1EA.4.1-2. The SS sends downlink MAC padding bits on the DL RMC. The transmission of PDSCH will make the UE send uplink ACK/NACK using PUCCH. There is no PUSCH transmission.

2.3	SS send appropriate TPC commands for PUCCH to the UE until the UE transmit PUCCH at PUMAX level.

2.4	Measure PUCCH EVM using Global In-Channel Tx-Test (Annex E) according to the UE’s declaration on the position of carrier centre frequency.

2.5	Send the appropriate TPC commands for PUCCH to the UE until the UE transmits PUCCH at -36.8dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -36.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

2.6	Measure PUCCH EVM using Global In-Channel Tx-Test (Annex E) according to the UE’s declaration on the position of carrier centre frequency.

Test procedure for PRACH:

3.1	The SS shall set RS EPRE according to Table 6.5.2.1EA.4.1-3.

3.2	PRACH is set according to Table 6.5.2.1EA.4.1-3.

3.3	The SS shall signal a Random Access Preamble ID via a MPDCCH order to the UE and initiate a Non-contention based Random Access procedure

3.4	The UE shall send the signalled preamble to the SS.

3.5	In response to the preamble, the SS shall transmit a random access response not corresponding to the transmitted random access preamble, or send no response.

3.6	The UE shall consider the random access response reception not successful then re-transmit the preamble with the calculated PRACH transmission power.

3.7	Repeat step 5 and 6 until the SS collect enough PRACH preambles (2 preambles for format 0 and 10 preambles for format 4). Measure the EVM in PRACH channel using Global In-Channel Tx-Test (Annex E) according to the UE’s declaration on the position of carrier centre frequency.

6.5.2.1EA.1.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the condition CEModeA and the following exceptions:

Table 6.5.2.1EA.4.3-1: PRACH-Config-DEFAULT: PRACH EVM measurement for FDD

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-7 PRACH-Config-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PRACH-ConfigInfo SEQUENCE { |  |  |  |
| prach-ConfigIndex | 4 |  |  |

Table 6.5.2.1EA.4.3-2: PRACH-ConfCommonDEFAULT: PRACH EVM measurement for TDD

| Derivation Path: TS 36.508 [7] clause 5.3.1, Table 5.3.1-3 PRACH-Config-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PRACH-ConfigInfo SEQUENCE { |  |  |  |
| prach-ConfigIndex | 53 |  |  |

Table 6.5.2.1EA.4.3-4: RACH-ConfigCommon-DEFAULT: PRACH EVM measurement

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-12 RACH-ConfigCommon-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon-DEFAULT ::= SEQUENCE { |  |  |  |
| preambleInfo SEQUENCE { |  |  |  |
| numberOfRA-Preambles | n52 |  |  |
| preamblesGroupAConfig SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| powerRampingParameters SEQUENCE { |  |  |  |
| powerRampingStep | dB0 |  |  |
| preambleInitialReceivedTargetPower | dBm-120 Test point 1 dBm-90 Test point 2 |  |  |
| } |  |  |  |
| ra-SupervisionInfo SEQUENCE { |  |  |  |
| preambleTransMax | n10 |  | FDD |
|  | n20 |  | TDD |
| ra-ResponseWindowSize | Sf10 |  |  |
| mac-ContentionResolutionTimer | sf48 |  |  |
| } |  |  |  |
| ra-SupervisionInfo SEQUENCE { |  |  |  |

Table 6.5.2.1EA.4.3-5: TDD-Config-DEFAULT: PRACH EVM measurement for TDD

| Derivation Path: TS 36.508 [7] clause 5.3.1, Table 5.3.1-1: TDD-Config-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-Config-DEFAULT ::= SEQUENCE { |  |  |  |
| subframeAssignment | sa1 |  |  |
| specialSubframePatterns | ssp5 | To enable two symbol UpPTS, and to have 9 symbols GP. |  |
| } |  |  |  |

6.5.2.1EA.1.5	Test requirement

The PUSCH EVM derived in E.4.2 shall not exceed 17,5 % for QPSK and BPSK, 12,5% for 16 QAM.

The PUSCH derived in E.4.6.2 shall not exceed [17,5 %] when embedded with data symbols of QPSK and BPSK, [12,5%] for 16 QAM.

The PUCCH EVM and derived in E.5.9.2 shall not exceed 17,5 %.

The PRACH EVM derived in FFS shall not exceed 17.5%.

##### 6.5.2.1EA.2	PUSCH-EVM with exclusion period for UE category M1

6.5.2.1EA.2.1	Test purpose

Same test purpose as in clause 6.5.2.1A.1

6.5.2.1EA.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 13 and forward of UE category M1.

6.5.2.1EA.2.3	Minimum conformance requirement

Same minimum conformance requirements as in clause 6.5.2.1A.3

6.5.2.1EA.2.4	Test description

6.5.2.1EA.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in sub-clause 5.2E. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.5.2.1EA.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and MPDCCH before measurement are specified in Annex C.2.

Table 6.5.2.1EA.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  | Normal Conditions |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 |  | Low range |  |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  | 5MHz |  |  |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |  |
|  | Downlink Configuration |  | Uplink Configuration |  |  |  |
| Ch BW | N/A |  | Mod'n | RB allocation |  |  |
|  |  |  |  | FDD and HD-FDD | TDD | Narrowband index1 |
| 5MHz |  |  | QPSK | Alternating 6 and 1 (as shown in Figure 6.5.2.1EA.2.4.2-1) and 6.5.2.1EA.2.4.2-2 | Alternating 6 and 1 (as shown in Figure 6.5.2.1EA.2.4.2-1) | 0 |
| 5MHz |  |  | 16 QAM | Alternating 5 and 1 (as shown in Figure 6.5.2.1EA.2.4.2-1) and 6.5.2.1EA.2.4.2-2 | Alternating 5 and 1 (as shown in Figure 6.5.2.1EA.2.4.2-1) | 0 |
| 5MHz |  |  | π/2-BPSK | Alternating 6 and ¼ PRB (as shown in Figure 6.5.2.1EA.2.4.2-1) and 6.5.2.1EA.2.4.2-2 (Note 2) | Alternating 6 and ¼ PRB (as shown in Figure 6.5.2.1EA.2.4.2-1) (Note 2) | 0 |
| 5MHz |  |  | QPSK | Alternating 5 and 1/2 (as shown in Figure 6.5.2.1EA.2.4.2-1) and 6.5.2.1EA.2.4.2-2 (Note 2) | Alternating 5 and 1/2 (as shown in Figure 6.5.2.1EA.2.4.2-1) (Note 2) | 0 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4. Note 2: Only applicable for UE supporting subPRB allocation. |  |  |  |  |  |  |

1.	Connect the SS to the UE to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508[7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to in Table 6.5.2.1EA.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [7] clause 5.2A.2AA. Message contents are defined in clause 6.5.2.1EA.4.3.

6.5.2.1EA.2.4.2	Test procedure

The test pattern is illustrated in figure 6.5.2.1EA.2.4.2-1 and 6.5.2.1EA.2.4.2-2.

.

Figure 6.5.2.1EA.2.4.2-1: Test pattern for FDD and TDD

Figure 6.5.2.1EA.2.4.2-2: Test pattern for HD-FDD

NOTE 1:	In TDD the free subframes are special subframes or DL, in FDD and HD-FDD the free subframes are OFF.

1.	SS sends uplink scheduling information for each UL HARQ process via MPDCCH DCI format 6-0A for C_RNTI to schedule the PUSCH. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC. The initial uplink RB allocation is 6 or 5. Send appropriate TPC commands for PUSCH to the UE to ensure the UE transmits PUSCH at 0dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or 0dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

2.	Schedule the UE's PUSCH data transmission as described in Figure 6.5.2.1EA.2.4.2-1 and 6.5.2.1EA.2.4.2-2 for 161) active time slots with an uplink RB allocation alternating pattern as described in table 6.5.2.1EA.2.4.1-1 while transmitting 0dB TPC command for PUSCH via the MPDCCH.

3.	Measure the EVM using Global In-Channel Tx-Test according to the UE’s declaration on the position of carrier centre frequency. The averaging across 161) timeslots is done across mixed RB allocations, as illustrated in Figure 6.5.2.1EA.2.4.2-1 and 6.5.2.1EA.2.4.2-2.

NOTE 1:	Averaging across 16 timeslots is used to represent each type of transition equally in the average.

6.5.2.1EA.2.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the condition CEModeA.

6.5.2.1EA.2.5	Test requirement

The PUSCH EVM derived in Annex E.4.2 taking into account Annex E.7 shall not exceed 17,5 % for QPSK and 12,5% for 16 QAM. The test requirements shall be fulfilled for early and late EVM window.

#### 6.5.2.1EB	Error Vector Magnitude (EVM) for UE category 1bis

##### 6.5.2.1EB.1	Test purpose

Same test purpose as in clause 6.5.2.1.

##### 6.5.2.1EB.2	Test applicability

This test applies to all types of E-UTRA UE release 13 and forward of UE category 1bis.

##### 6.5.2.1EB.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.2.1.3.

##### 6.5.2.1EB.4	Test description

6.5.2.1EB.4.1	Initial conditions

Same initial conditions as in clause 6.5.2.1.4.1 with the following exception:

-	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.3 using only main UE Tx/Rx antenna.

6.5.2.1EB.4.2	Test procedure

Same test procedure as in clause 6.5.2.1.4.2.

6.5.2.1EB.4.3	Message contents

Same message contents as in clause 6.5.2.1.4.3.

##### 6.5.2.1EB.5	Test requirement

The PUSCH EVM derived in E.4.2 shall not exceed 17,5 % for QPSK and BPSK, 12,5% for 16 QAM.

The PUSCH derived in E.4.6.2 shall not exceed 17,5 % when embedded with data symbols of QPSK and BPSK, 12,5% for 16 QAM.

The PUCCH EVM and derived in E.5.9.2 shall not exceed 17,5 %.

The PRACH EVM derived in FFS shall not exceed 17.5%.

#### 6.5.2.1EC	Error Vector Magnitude (EVM) for UE category M2

##### 6.5.2.1EC.1	Error Vector Magnitude (EVM) for UE category M2

6.5.2.1EC.1.1	Test Purpose

Same test purpose as in clause 6.5.2.1.1.

6.5.2.1EC.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 14 and forward of UE category M2.

6.5.2.1EC.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.2.1.3.

6.5.2.1EC.1.4	Test description

6.5.2.1EC.1.4.1		Initial conditions

Same as in clause 6.5.2.1EA.1.4.1 with the following exceptions:

-	Instead of Table 6.5.2.1EA.1.4.1-1  use Table 6.5.2.1EC.1.4.1-1

-	Instead of Table 6.5.2.1EA.1.4.1-2  use Table 6.5.2.1EC.1.4.1-2

-	Instead of Table 6.5.2.1EA.1.4.1-3  use Table 6.5.2.1EC.1.4.1-3

Table 6.5.2.1EC.1.4.1-1: Test Configuration Table for PUSCH

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  | NC |  |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  | Low range, Mid range, High range |  |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  | 5MHz |  |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  | Downlink Configuration | Uplink Configuration |  |  |  |
| Ch BW | N/A for PUSCH EVM testing | Mod'n | RB allocation |  |  |
|  |  |  | FDD and HD-FDD | TDD | Narrowband index1 |
| 5MHz |  | QPSK | 24 | 24 | 0 |
| 5MHz |  | QPSK | 1 (Note 3), 3 (Note 4) | 1 (Note 3), 3 (Note 4) | 0 |
| 5MHz |  | 16QAM | 24 | 24 | 0 |
| 5MHz |  | 16QAM | 1 (Note 3), 3 (Note 4) | 1 (Note 3), 3 (Note 4) | 0 |
| Note 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.4. Note 2: The RBstart of partial RB allocation shall be RB#0 and RB# (6 – (RB allocation mod 6)) of the narrowband. Note 3: Only applicable for Power class 3 Note 4: Only applicable for Power class 5 |  |  |  |  |  |

Table 6.5.2.1EC.1.4.1-2: Test Configuration Table for PUCCH

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  | NC |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in |  |  | 5MHz |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
|  | Downlink Configuration |  |  |  | Uplink Configuration |
| Ch BW | Mod'n | RB allocation |  |  | FDD and HD-FDD: PUCCH format = Format 1a TDD: PUCCH format = Format 1a / 1b |
|  |  | FDD | TDD | Narrowband index2 |  |
| 5MHz | QPSK | 16 | 16 | 0 |  |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.4. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 – (RB allocation mod 6)) of the narrowband. |  |  |  |  |  |

Table 6.5.2.1EC.1.4.1-3: Test Configuration for PRACH

| Initial Conditions |  |  |
| --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) | NC |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) | Low range, Mid range, High range |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) | 5MHz, 15MHz |  |
| PRACH preamble format |  |  |
|  | FDD and HD-FDD | TDD |
| PRACH Configuration Index | 4 | 53 |
| RS EPRE setting for test point 1 (dBm/15kHz) | -71 | -63 |
| RS EPRE setting for test point 2 (dBm/15kHz) | -86 | -78 |

6.5.2.1EC.1.4.3	Message contents

Same as in clause 6.5.2.1EA.1.4.3.

6.5.2.1EC.1.5	Test requirement

Same as in clause 6.5.2.1EA.1.5.

##### 6.5.2.1EC.2	PUSCH-EVM with exclusion period for UE category M2

6.5.2.1EC.2.1	Test purpose

Same test purpose as in clause 6.5.2.1A.1

6.5.2.1EC.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 14 and forward of UE category M2.

6.5.2.1EC.2.3	Minimum conformance requirement

Same minimum conformance requirements as in clause 6.5.2.1A.3

6.5.2.1EC.2.4	Test description

6.5.2.1EC.2.4.1	Initial conditions

Same initial condition as in clause 6.5.2.1EA.2.4.1 with the following exception:

-	Instead of Table 6.5.2.1EA.2.4.1-1 -> use Table 6.5.2.1EC.2.4.1-1

Table 6.5.2.1EC.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | Normal Conditions |  |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 |  |  |  | Low range |  |  |  |  |  |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  |  | 5MHz |  |  |  |  |  |  |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Downlink Configuration |  |  | Uplink Configuration |  |  |  |  |  |  |  |
| Ch BW |  | N/A |  |  | Mod'n |  | RB allocation |  |  |  |  |  |
|  |  |  |  |  |  |  | FDD and HD-FDD |  | TDD |  | Narrowband index1 |  |
| 5MHz |  |  |  |  | QPSK |  | Alternating 12 and 1 (as shown in Figure 6.5.2.1EC.2.4.2-1) and 6.5.2.1EC.2.4.2-2 |  | Alternating 12 and 1 (as shown in Figure 6.5.2.1EC.2.4.2-1) |  | 0 |  |
| 5MHz |  |  |  |  | 16 QAM |  | Alternating 12 and 1 (as shown in Figure 6.5.2.1EC.2.4.2-1) and 6.5.2.1EC.2.4.2-2 |  | Alternating 12 and 1 (as shown in Figure 6.5.2.1EC.2.4.2-1) |  | 0 |  |
| 5MHz |  |  |  |  | π/2-BPSK |  | Alternating 12 and ¼ (as shown in Figure 6.5.2.1EC.2.4.2-1) and 6.5.2.1EC.2.4.2-2 (Note 2) |  | Alternating 12 and ¼ (as shown in Figure 6.5.2.1EC.2.4.2-1) (Note 2) |  | 0 |  |
| 5MHz |  |  |  |  | QPSK |  | Alternating 12 and 1/2 (as shown in Figure 6.5.2.1EC.2.4.2-1) and 6.5.2.1EC.2.4.2-2 (Note 2) |  | Alternating 12 and 1/2 (as shown in Figure 6.5.2.1EC.2.4.2-1) (Note 2) |  | 0 |  |
| Note 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.4. Note 2: Only applicable for UE supporting subPRB allocation |  |  |  |  |  |  |  |  |  |  |  |  |

6.5.2.1EC.2.4.2	Test procedure

Same test procedure as in clause 6.5.2.1EA.2.4.2 with the following exceptions:

-	Instead figure 6.5.2.1EA.2.4.2-1 -> use figure 6.5.2.1EC.2.4.2-1

-	Instead figure 6.5.2.1EA.2.4.2-2 -> use figure 6.5.2.1EC.2.4.2-2

-	Instead of 6/5 RBs -> use 12 RBs

.

Figure 6.5.2.1EC.2.4.2-1: Test pattern for FDD and TDD

Figure 6.5.2.1EC.2.4.2-2: Test pattern for HD-FDD

6.5.2.1EC.2.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the condition CEModeA.

6.5.2.1EC.2.5	Test requirement

Same test requirement as in clause 6.5.2.1EA.2.5.

#### 6.5.2.1F.1	Error Vector Magnitude (EVM) for category NB1 and NB2

6.5.2.1F.1.1	Test Purpose

The Error Vector Magnitude is a measure of the difference between the reference waveform and the measured waveform. This difference is called the error vector. Before calculating the EVM the measured waveform is corrected by the sample timing offset and RF frequency offset. Then the carrier leakage shall be removed from the measured waveform before calculating the EVM.

The measured waveform is further modified by selecting the absolute phase and absolute amplitude of the Tx chain. The EVM result is defined after the front-end IDFT as the square root of the ratio of the mean error vector power to the mean reference power expressed as a %.

6.5.2.1F.1.2	Test applicability

This test case applies to all types of E-UTRA FDD UE release 13 and forward of category NB1.

This test case applies to all types of E-UTRA FDD UE release 14 and forward of category NB2.

This test case applies to all types of E-UTRA TDD UE release 15 and forward of category NB1 and NB2.

6.5.2.1F.1.3	Minimum conformance requirements

The RMS average of the basic EVM measurements for 240/LCtone slots excluding any transient period for the average EVM case, where LCtone = {1, 3, 6, 12} is the number of subcarriers for the NB-IoT transmission, for the different modulations schemes shall not exceed the values specified in Table 6.5.2.1F.1.3-1 for the parameters defined in Table 6.5.2.1F.1.3-2. For EVM evaluation purposes, both NPRACH formats are considered to have the same EVM requirement as QPSK modulated.

Table 6.5.2.1F.1.3-1: Minimum requirements for Error Vector Magnitude

| Parameter | Unit | Average EVM Level | Reference Signal EVM Level |
| --- | --- | --- | --- |
| BPSK or QPSK | % | 17.5 | 17.5 |

Table 6.5.2.1F.1.3-2: Parameters for Error Vector Magnitude

| Parameter | Unit | Level |
| --- | --- | --- |
| UE Output Power | dBm | -40 |
| Operating conditions |  | Normal conditions |

The normative reference for this requirement is TS 36.101 [2] clause 6.5.2F.1.

##### 6.5.2.1F.1.4	Test description

6.5.2.1F.1.4.1		Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions and test frequencies based on the subset of E-UTRA operating bands defined for NB-IoT in clause 5.2F. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.5.2.1F.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2.4. Configurations of NPDSCH and NPDCCH before measurement are specified in Annex C.2.

Table 6.5.2.1F.1.4.1-1: Test Configuration for NPUSCH for FDD & TDD

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 8.1.1 |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 8.1.3.1 |  | Frequency ranges defined in Annex K.1.1 |  |  |
| Test Parameters |  |  |  |  |
| Configuration ID | Downlink Configuration | Uplink Configuration |  |  |
|  | N/A | Modulation | Ntones, start position | Sub-carrier spacing (kHz) |
| 1 |  | QPSK | 1@0 | 3.75 |
| 2 |  | QPSK | 1@47 | 3.75 |
| 3 |  | QPSK | 1@0 | 15 |
| 4 |  | QPSK | 1@11 | 15 |
| 5 (Note 1) |  | QPSK | 12@0 | 15 |
| Note 1: Applicable to UE supporting UL multi-tone transmissions |  |  |  |  |

Table 6.5.2.1F.1.4.1-2: Test Configuration for NPRACH for FDD & TDD

| Initial Conditions |  |
| --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 8.1.1) | Normal |
| Test Frequencies (as specified in TS36.508 [7] subclause 8.1.3.1) | Frequency ranges defined in Annex K.1.1 |
| NPRACH preamble format | 1 |
| NRS EPRE setting for test point (dBm/15kHz) | -110 |

1.	Connect the SS to the UE to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508[7] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex H.

4.	The UL Reference Measurement channels are set according to in Table 6.5.2.1F.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5. Message contents are defined in clause 6.5.2.1F.1.4.3.

6.5.2.1F.1.4.2	Test procedure

Test procedure for NPUSCH:

1.1. SS sends uplink scheduling information for UL HARQ process via NPDCCH DCI format N0 for C_RNTI to schedule the UL RMC according to Table 6.5.2.1F.1.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

1.2	Configure the UE to transmit at PUMAX level.

1.3	Measure the EVM and  using Global In-Channel Tx-Test (Annex E). The measurement period of EVM for 240/Ntones slots should exclude any transient period for the average EVM case, where Ntones = {1, 3, 6, 12} is the number of subcarriers for the NB-IoT transmission.

1.4 Release the connection through State 3A-NB.

1.5 Modify system information elements according to Table 6.5.2.1F.1.4.3-1 and Table 6.5.2.1F.1.4.3-2 and notify the UE via paging message with SystemInformationModification included (test point 2).

1.6 Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5 using the new UL power control setting.

1.7 SS sends uplink scheduling information for UL HARQ process via NPDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.5.2.1F.1.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

1.8	Measure the EVM and  using Global In-Channel Tx-Test (Annex E). The measurement period of EVM for 240/Ntones slots should exclude any transient period for the average EVM case, where Ntones = {1, 3, 6, 12} is the number of subcarriers for the NB-IoT transmission.

Test procedure for NPRACH:

2.1	The SS shall set RS EPRE according to Table 6.5.2.1F.1.4.1-2.

2.2	NPRACH is set according to Table 6.5.2.1F.1.4.1-2.

2.3	The UE shall send a preamble to the SS.

2.4	In response to the preamble, the SS shall transmit a random access response not corresponding to the transmitted random access preamble, or send no response.

2.5	The UE shall consider the random access response reception not successful then re-transmit the preamble with the calculated NPRACH transmission power.

2.6	Repeat step 4 and 5 until the SS collect enough NPRACH preambles (64 preambles). Measure the EVM in NPRACH channel using Global In-Channel Tx-Test (Annex E).

NOTE 1:	For configuration IDs applicable to UE depending on UE capability in Test Configuration Table with different UL sub-carrier spacing, the SS shall release the connection through State 3A-NB and finally ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5 using the appropriate UL subcarrier spacing in Random Access Response message.

6.5.2.1F.1.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 8.1.6 with the following exceptions:

Table 6.5.2.1F.1.4.3-1: P0-NominalNPUSCH-r13 configuration for test point 2

| Derivation Path: TS 36.508 [7] clause 8.1.6.3 Table 8.1.6.3-14: UplinkPowerControlCommon-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| p0-NominalNPUSCH-r13 | -117 (dBm) |  |  |
| alpha-r13 | al1 (1) |  |  |
| deltaPreambleMsg3-r13 | 4 |  |  |
| } |  |  |  |

Table 6.5.2.1F.1.4.3-2: NPDSCH-ConfigCommon-NB-DEFAULT configuration for test point 2

| Derivation Path: TS 36.508 [7] clause 8.1.6.3 Table 8.1.6.3-4: NPDSCH-ConfigCommon-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPDSCH-ConfigCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| nrs-Power-r13 | 21 (dBm) |  |  |
| } |  |  |  |

Table 6.5.2.1F.1.4.3-3: RACH-ConfigCommon-NB-DEFAULT NPRACH EVM Measurement

| Derivation Path: TS 36.508 [7] clause 8.1.6.3, Table 8.1.6.3-8 RACH-ConfigCommon-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| powerRampingParameters-r13 SEQUENCE { |  |  |  |
| powerRampingStep | dB0 | 0 dB |  |
| } |  |  |  |
| ) |  |  |  |

Table 6.5.2.1F.1.4.3-4: NPRACH-ConfigSIB-NB-DEFAULT for NPRACH EVM Measurement

| Derivation Path: TS 36.508 [7] clause 8.1.6.3, Table 8.1.6.3-5 NPRACH-ConfigSIB-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPRACH-ConfigSIB-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| nprach-CP-Length-r13 | us266dot7 | 8192*Ts |  |
| } |  |  |  |

##### 6.5.2.1F.1.5	Test requirement

The NPUSCH EVM derived in E.8.1 shall not exceed 17,5% for BPSK and QPSK.

The NPUSCH derived in E.8.2 shall not exceed 17,5 % when embedded with data symbols of BPSK and QPSK.

The NPRACH EVM derived in E.8.3 shall not exceed 17.5%.

#### 6.5.2.1G	Error Vector Magnitude (EVM) for V2X Communication

##### 6.5.2.1G.0	Minimum conformance requirements

For V2X physical channels PSCCH, PSSCH and PSBCH, the Error Vector Magnitude requirements shall be as specified for PUSCH in subclause 6.5.2.1 for the corresponding modulation and transmission bandwidth.

For V2X sidelink physical channels PSCCH, PSSCH and PSBCH, the Error Vector Magnitude requirements shall be as specified separately for PSSCH and PSCCH for the corresponding modulation and transmission bandwidth. The measurement period for EVM of PSSCH and PSCCH is 15 subframes. The measurement period for reference signal EVM is 30 subframes. When V2X transmissions are shortened due to transmission gap of 1 symbol at the end of the subframe, the EVM measurement interval is reduced by one symbol, accordingly.

For PSBCH the duration over which EVM is averaged shall be 24 subframes.

For intra-band contiguous multi-carrier operation the EVM requirement shall apply for each component carrier.

When UE is configured for simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions for inter-band E-UTRA V2X / E-UTRA bands specified in Table 5.2G-2, the requirements in subclause 6.5.2.1G.0 apply for V2X sidelink transmission and the requirements in subclause 6.5.2.1.3 apply for the E-UTRA uplink transmission.

The normative reference for this requirement is TS 36.101 [2] clause 6.5.2G and 6.5.2G.1.

##### 6.5.2.1G.1	Error Vector Magnitude (EVM) for V2X Communication / Non-concurrent with E-UTRA uplink transmissions

###### 6.5.2.1G.1.1	Test Purpose

The Error Vector Magnitude is a measure of the difference between the reference waveform and the measured waveform. This difference is called the error vector. Before calculating the EVM the measured waveform is corrected by the sample timing offset and RF frequency offset. Then the carrier leakage shall be removed from the measured waveform before calculating the EVM.

The measured waveform is further modified by selecting the absolute phase and absolute amplitude of the Tx chain. The EVM result is defined after the front-end IDFT as the square root of the ratio of the mean error vector power to the mean reference power expressed as a %.

###### 6.5.2.1G.1.2	Test applicability

This test case applies to all types of UE that supports V2X Sidelink communication and Band 47.

###### 6.5.2.1G.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5.2.1G.0.

###### 6.5.2.1G.1.4	Test description

6.5.2.1G.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidth based on E-UTRAN operating bands specified in subclause 5.4.2G. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.5.2.1G.1.4.1-1. The details of the V2X reference measurement channels (RMCs) are specified in Annex A.8.3 and the GNSS configuration in TS 36.508 [7] subclause 4.11.

Table 6.5.2.1G.1.4.1-1: Test Configuration Table for PSSCH and PSCCH

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |
|  | V2X Configuration to Transmit |  |  |
| Ch BW | Modulation |  | PSSCH RB allocation |
| 10 MHz | QPSK |  | 48@2 |
| 10 MHz | 16QAM |  | 48@2 |
| 20 MHz | QPSK |  | 96@2 |
| 20 MHz | 16QAM |  | 96@2 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in specified in subclause 5.4.2G. |  |  |  |

Table 6.5.2.1G.1.4.1-2: Test Configuration Table for PSBCH

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |
|  | PSBCH Configuration |  |  |
| Ch BW | Modulation |  | PSBCH RB allocation |
| 10 MHz | QPSK |  | 6 |
| 20 MHz | QPSK |  | 6 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in specified in subclause 5.4.2G. |  |  |  |

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 36.508 [7] Figure A.92.

2.	The parameter settings for the V2X sidelink transmission over PC5 for both SS and UE are pre-configured according to TS 36.508 [7] subclause 4.10.1. Message content exceptions are defined in clause 6.5.2.1G.1.4.3.

3.	The V2X reference measurement channel is set according to Table 6.5.2G.1.4.1-1.

4.	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS36.508 [7] Table 4.11.2-2. Geographical area #1 is also pre-configured in the UE.

5.	Propagation conditions are set according to Annex B.0;

6.	Ensure the UE is in State 5A-V2X in Transmit Mode according to TS 36.508 [7] clause 4.5.9.

7.	The GNSS simulator is triggered to start step 1 of Scenario #1 to simulate a location in the centre of Geographical area #1. Wait for the UE to acquire the GNSS signal and start to transmit.

6.5.2.1G.1.4.2	Test procedure

Test procedure for PSSCH and PSCCH:

1.1	The UE starts to perform the V2X sidelink communication according to SL-V2X-Preconfiguration. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the V2X RMC.

1.2	Configure the UE to transmit at PUMAX level.

1.3	Measure the EVM using Global In-Channel Tx-Test (Annex E). The measurement period is 15 subframes. When V2X transmissions are shortened due to transmission gap of 1 symbol at the end of the subframe, the EVM measurement interval is reduced by one symbol, accordingly.

1.4	Modify SL-V2X-Preconfiguration to ensure the UE to transmit at a relative low power, according to Table 6.5.2.1G.1.4.3-2.

1.5	Ensure the UE is in State 5A-V2X in Transmit Mode according to TS 36.508 [7] clause 4.5.9 using the new UL power control setting.

1.6	The UE starts to perform the V2X sidelink communication according to SL-V2X-Preconfiguration. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the V2X RMC.

1.7	Measure the EVM using Global In-Channel Tx-Test (Annex E). The measurement period is 15 subframes. When V2X transmissions are shortened due to transmission gap of 1 symbol at the end of the subframe, the EVM measurement interval is reduced by one symbol, accordingly.

Test procedure for PSBCH:

2.1	The UE starts to perform the V2X PSBCH/SLSS transmission according to SL-V2X-Preconfiguration every 40 subframes. The synchronization configuration of UE is according to Table 6.5.2.1G.1.4.3-3.

2.2	Configure the UE to transmit at PUMAX level.

2.3	Measure the EVM using Global In-Channel Tx-Test (Annex E) until the SS collect 24 subframes of PSBCH. When V2X transmissions are shortened due to transmission gap of 1 symbol at the end of the subframe, the EVM measurement interval is reduced by one symbol, accordingly.

2.4	Modify SL-V2X-Preconfiguration to ensure the UE to transmit at a relative low power, according to Table 6.5.2.1G.1.4.3-3 and Table 6.5.2.1G.1.4.3-4.

2.5	Ensure the UE is in State 5A-V2X in Transmit Mode according to TS 36.508 [7] clause 4.5.9 using the new UL power control setting.

2.6	The UE starts to perform the V2X PSBCH/SLSS transmission according to SL-V2X-Preconfiguration every 40 subframes.

2.7	Measure the EVM using Global In-Channel Tx-Test (Annex E) until the SS collect 24 subframes of PSBCH. When V2X transmissions are shortened due to transmission gap of 1 symbol at the end of the subframe, the EVM measurement interval is reduced by one symbol, accordingly.

6.5.2.1G.1.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6, 4.7.I and 4.10 with the following exceptions:

Table 6.5.2.1G.1.4.3-1: Void

Table 6.5.2.1G.1.4.3-2: maxTxPower configuration for PSSCH/PSCCH test point 2

| Derivation Path: 36.508 Table 4.6.3-20K |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-V2X-PreconfigFreqInfo-r14-DEFAULT ::= SEQUENCE { |  |  |  |
| v2x-CommPreconfigGeneral-r14 SEQUENCE { |  |  |  |
| maxTxPower-r12 | -30 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.2.1G.1.4.3-3: synConfig configuration for PSBCH test point 1 and 2

| Derivation Path: 36.508 Table 4.6.3-20K |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-V2X-PreconfigFreqInfo-r14-DEFAULT ::= SEQUENCE { |  |  |  |
| v2x-CommPreconfigSync-r14 SEQUENCE { |  |  |  |
| syncOffsetIndicators-r14 SEQUENCE { |  |  |  |
| syncOffsetIndicator1-r14 | 0 |  |  |
| syncOffsetIndicator2-r14 | 50 |  |  |
| syncOffsetIndicator3-r14 | Not present |  |  |
| } |  |  |  |
| syncTxParameters-r14 | 31dBm |  |  |
| syncTxThreshOoC-r14 | 4 | -90dBm |  |
| filterCoefficient-r14 | fc2 |  |  |
| syncRefMinHyst-r14 | dB6 |  |  |
| syncRefDiffHyst-r14 | dB6 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.2.1G.1.4.3-4: maxTxPower configuration for PSBCH test point 2

| Derivation Path: 36.508 Table 4.6.3-20K |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-V2X-PreconfigFreqInfo-r14-DEFAULT ::= SEQUENCE { |  |  |  |
| v2x-CommPreconfigGeneral-r14 SEQUENCE { |  |  |  |
| maxTxPower-r12 | -30 |  |  |
| } |  |  |  |
| } |  |  |  |

###### 6.5.2.1G.1.5	Test requirement

The PSSCH EVM derived in E.9 shall not exceed 17.5% for QPSK, 12.5% for 16 QAM.

The PSCCH EVM and derived in E.9 shall not exceed 17.5%.

The PSBCH EVM derived in FFS shall not exceed 17.5%.

##### 6.5.2.1G.2	Error Vector Magnitude (EVM) for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions

###### 6.5.2.1G.2.1	Test Purpose

The Error Vector Magnitude is a measure of the difference between the reference waveform and the measured waveform. This difference is called the error vector. Before calculating the EVM the measured waveform is corrected by the sample timing offset and RF frequency offset. Then the carrier leakage shall be removed from the measured waveform before calculating the EVM.

The measured waveform is further modified by selecting the absolute phase and absolute amplitude of the Tx chain. The EVM result is defined after the front-end IDFT as the square root of the ratio of the mean error vector power to the mean reference power expressed as a %.

###### 6.5.2.1G.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 14 and forward that support simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions for inter-band E-UTRA V2X / E-UTRA bands specified in Table 5.2G-2.

###### 6.5.2.1G.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5.2.1G.0.

###### 6.5.2.1G.2.4	Test description

6.5.2.1G.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on Inter-band con-current V2X configurations specified in Table 5.4.2G.1-2. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 6.5.2.1G.2.4.1-1 to Table 6.5.2.1G.2.4.1-2. The details of the uplink and V2X reference measurement channels (RMCs) are specified in Annexes A.2 and A.8.3 respectively. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.2.1G.2.4.1-1: Test Configuration Table for PUSCH, PSSCH and PSCCH

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | NC |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |  |
| Test Channel Bandwidths as specified in 5.4.2G |  |  | Lowest NRB_agg, Highest NRB_agg |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  | Downlink | Uplink Configuration |  | V2X Configuration |  |  |
| Ch BW | N/A for EVM test case | Mod'n | RB allocation | Ch BW | Mod'n | PSSCH RB allocation |
| 1.4MHz |  | QPSK | 6 | 10 MHz | QPSK | 48@2 |
| 1.4MHz |  | QPSK | 1 | 10 MHz | 16QAM | 48@2 |
| 1.4MHz |  | 16QAM | 6 | 20 MHz | QPSK | 96@2 |
| 1.4MHz |  | 16QAM | 1 | 20 MHz | 16QAM | 96@2 |
| 3MHz |  | QPSK | 15 |  |  |  |
| 3MHz |  | QPSK | 4 |  |  |  |
| 3MHz |  | 16QAM | 15 |  |  |  |
| 3MHz |  | 16QAM | 4 |  |  |  |
| 5MHz |  | QPSK | 25 |  |  |  |
| 5MHz |  | QPSK | 8 |  |  |  |
| 5MHz |  | 16QAM | 25 |  |  |  |
| 5MHz |  | 16QAM | 8 |  |  |  |
| 10MHz |  | QPSK | 50 |  |  |  |
| 10MHz |  | QPSK | 12 |  |  |  |
| 10MHz |  | 16QAM | 50 (Note 3) |  |  |  |
| 10MHz |  | 16QAM | 12 |  |  |  |
| 15MHz |  | QPSK | 75 |  |  |  |
| 15MHz |  | QPSK | 16 |  |  |  |
| 15MHz |  | 16QAM | 75 (Note 3) |  |  |  |
| 15MHz |  | 16QAM | 16 |  |  |  |
| 20MHz |  | QPSK | 100 |  |  |  |
| 20MHz |  | QPSK | 18 |  |  |  |
| 20MHz |  | 16QAM | 100 (Note 3) |  |  |  |
| 20MHz |  | 16QAM | 18 |  |  |  |
| Note 1: V2X con-current band configurations are checked separately for each band configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2G.1-2. Note 2: For partial RB allocation of E-UTRA carrier, the RBstart shall be RB #0 and RB# (max+1 - RB allocation) of the channel bandwidth. Note 3: Applies only for UE-Categories ≥2 |  |  |  |  |  |  |

Table 6.5.2.1G.2.4.1-2: Test Configuration Table for PSBCH

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | NC |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |  |
| Test Channel Bandwidths as specified in 5.4.2G |  |  | Lowest NRB_agg, Highest NRB_agg |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  | Downlink | Uplink Configuration |  | V2X Configuration PSBCH |  |  |
| Ch BW | N/A for EVM test case | Mod'n | RB allocation | Ch BW | Mod'n | RB allocation |
| 1.4MHz |  | QPSK | 6 | 10 MHz | QPSK | 6 |
| 1.4MHz |  | QPSK | 1 | 20 MHz | QPSK | 6 |
| 1.4MHz |  | 16QAM | 6 |  |  |  |
| 1.4MHz |  | 16QAM | 1 |  |  |  |
| 3MHz |  | QPSK | 15 |  |  |  |
| 3MHz |  | QPSK | 4 |  |  |  |
| 3MHz |  | 16QAM | 15 |  |  |  |
| 3MHz |  | 16QAM | 4 |  |  |  |
| 5MHz |  | QPSK | 25 |  |  |  |
| 5MHz |  | QPSK | 8 |  |  |  |
| 5MHz |  | 16QAM | 25 |  |  |  |
| 5MHz |  | 16QAM | 8 |  |  |  |
| 10MHz |  | QPSK | 50 |  |  |  |
| 10MHz |  | QPSK | 12 |  |  |  |
| 10MHz |  | 16QAM | 50 (Note 3) |  |  |  |
| 10MHz |  | 16QAM | 12 |  |  |  |
| 15MHz |  | QPSK | 75 |  |  |  |
| 15MHz |  | QPSK | 16 |  |  |  |
| 15MHz |  | 16QAM | 75 (Note 3) |  |  |  |
| 15MHz |  | 16QAM | 16 |  |  |  |
| 20MHz |  | QPSK | 100 |  |  |  |
| 20MHz |  | QPSK | 18 |  |  |  |
| 20MHz |  | 16QAM | 100 (Note 3) |  |  |  |
| 20MHz |  | 16QAM | 18 |  |  |  |
| Note 1: V2X con-current band configurations are checked separately for each band configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2G.1-2. Note 2: For partial RB allocation of E-UTRA carrier, the RBstart shall be RB #0 and RB# (max+1 - RB allocation) of the channel bandwidth. Note 3: Applies only for UE-Categories ≥2 |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.93a.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL and V2X reference measurement channels are set according to Table 6.5.2.1G.2.4.1-1 to Table 6.5.2.1G.2.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF-V2X according to TS 36.508 [7] clause 5.2A.2C. Message content exceptions are defined in clause 6.5.2.1G.2.4.3.

6.5.2.1G.2.4.2	Test procedure

Test procedure for PUSCH, PSSCH and PSCCH:

1.1 The SS simultaneously sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC and V2X scheduling information via PDCCH DCI format 5A for SL_V_RNTI to schedule the V2X RMC according to Table 6.5.2.1G.2.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL and V2X RMCs.

1.2	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE to ensure that the UE transmits at PUMAX level for the duration of the test. Configure the UE to transmit PSCCH and PSSCH with the PUMAX level.

1.3	Measure the EVM using Global In-Channel Tx-Test (Annex E). The measurement period for EVM of PUSCH is according to 6.5.2.1. The measurement period for EVM of PSSCH and PSCCH is according to 6.5.2.1G.1.

1.4	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE uplink output power is –36.8dBm ±3.2dB for carrier frequency f ≤ 3.0GHz. Configure the UE to transmit PSCCH and PSSCH with a relative low power according to Table 6.5.2.1G.2.4.3-2.

1.5	Ensure the UE is in State 3A-RF-V2X according to TS 36.508 [7] clause 5.2A.2C using the new UL power control setting.

1.6 The SS simultaneously sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC and V2X scheduling information via PDCCH DCI format 5A for SL_V_RNTI to schedule the V2X RMC according to Table 6.5.2.1G.2.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL and V2X RMCs.

1.7Measure the EVM using Global In-Channel Tx-Test (Annex E). The measurement period for EVM of PUSCH is according to 6.5.2.1. The measurement period for EVM of PSSCH and PSCCH is according to 6.5.2.1G.1.

Test procedure for PUSCH and PSBCH:

2.1	The UE starts to perform the V2X PSBCH/SLSS transmission according to SL-V2X-Configuration every 40 subframes. The synchronization configuration of UE is according to Table 6.5.2.1G.2.4.3-3.

2.2	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE to ensure that the UE transmits at PUMAX level for the duration of the test. Configure the UE to transmit PSBCH with the PUMAX level.

2.3	Measure the EVM using Global In-Channel Tx-Test (Annex E). The measurement period for EVM of PUSCH is according to 6.5.2.1. The measurement period for EVM of PSBCH is according to 6.5.2.1G.1.

2.4	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE uplink output power is –36.8dBm ±3.2dB for carrier frequency f ≤ 3.0GHz. Configure the UE to transmit PSBCH with a relative low power according to Table 6.5.2.1G.2.4.3-3 and Table 6.5.2.1G.2.4.3-4.

2.5	Ensure the UE is in State 3A-RF-V2X according to TS 36.508 [7] clause 5.2A.2C using the new UL power control setting.

2.6	The UE starts to perform the V2X PSBCH/SLSS transmission according to SL-V2X-Configuration every 40 subframes.

2.7	Measure the EVM using Global In-Channel Tx-Test (Annex E). The measurement period for EVM of PUSCH is according to 6.5.2.1. The measurement period for EVM of PSBCH is according to 6.5.2.1G.1.

6.5.2.1G.2.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the following exceptions:

Table 6.5.2.1G.2.4.3-1: SL-CommResourcePoolV2X-r14 configuration for PSSCH/PSCCH test point 1 and 2

| Derivation Path: 36.508 [7] Table 4.6.3-20C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-CommResourcePoolV2X-r14-DEFAULT ::= SEQUENCE { |  |  |  |
| sizeSubchannel-r14 | n50 |  | BW10 |
|  | n100 |  | BW20 |
| numSubchannel-r14 | n1 |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| BW10 | 10 MHz channel bandwidth cell environment |
| BW20 | 20 MHz channel bandwidth cell environment |

Table 6.5.2.1G.2.4.3-2: sl-MaxTxPower configuration for PSSCH/PSCCH test point 2

| Derivation Path: 36.508 [7] Table 4.6.3-20E |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-InterFreqInfoV2X-r14-DEFAULT ::= SEQUENCE { |  |  |  |
| sl-MaxTxPower-r14 | -30 |  |  |
| } |  |  |  |

Table 6.5.2.1G.2.4.3-3: synConfig configuration for PSBCH test point 1 and 2

| Derivation Path: 36.508 [7] Table 4.6.3-20I |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-V2X-InterFreqUE-Config-r14-DEFAULT ::= SEQUENCE { |  |  |  |
| v2x-SyncConfig-r14 SEQUENCE { |  |  |  |
| asyncParameters-r13 SEQUENCE { |  |  |  |
| syncOffsetIndicator-r12 | 0 |  |  |
| } |  |  |  |
| syncOffsetIndicator2-r14 | 50 |  |  |
| syncOffsetIndicator3-r14 | Not present |  |  |
| txParameters-r13 SEQUENCE { |  |  |  |
| syncTxParameters-r13 |  |  |  |
| alpha-r12 | al0 |  |  |
| p0-r12 | 31 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.2.1G.2.4.3-4: sl-MaxTxPower configuration for PSBCH test point 2

| Derivation Path: 36.508 [7] Table 4.6.3-20E |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-InterFreqInfoV2X-r14-DEFAULT ::= SEQUENCE { |  |  |  |
| sl-MaxTxPower-r14 | -30 |  |  |
| } |  |  |  |

###### 6.5.2.1G.2.5	Test requirement

The PUSCH EVM derived in E.4.2 shall not exceed 17.5 % for QPSK, 12.5% for 16 QAM.

The PUSCH derived in E.4.6.2 shall not exceed 17.5 % when embedded with data symbols of QPSK, 12.5% for 16 QAM.

The PSSCH EVM derived in E.9.1 shall not exceed 17.5% for QPSK, 12.5% for 16 QAM.

The PSCCH EVM derived in E.9.1 shall not exceed 17.5%.

The PSBCH EVM derived in E.9.2 shall not exceed 17.5%.

##### 6.5.2.1G.3	Error Vector Magnitude (EVM) for V2X Communication / Intra-band contiguous multi-carrier operation

###### 6.5.2.1G.3.1	Test Purpose

The Error Vector Magnitude is a measure of the difference between the reference waveform and the measured waveform. This difference is called the error vector. Before calculating the EVM the measured waveform is corrected by the sample timing offset and RF frequency offset. Then the carrier leakage shall be removed from the measured waveform before calculating the EVM.

The measured waveform is further modified by selecting the absolute phase and absolute amplitude of the Tx chain. The EVM result is defined after the front-end IDFT as the square root of the ratio of the mean error vector power to the mean reference power expressed as a %.

###### 6.5.2.1G.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 14 and forward that support Intra-band contiguous multi-carrier operation V2X Communication.

###### 6.5.2.1G.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5.2.1G.0.

###### 6.5.2.1G.3.4	Test description

6.5.2.1G.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on V2X intra-band contiguous multi-carrier operation bands specified in Table 5.4.2G.1-4. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 6.5.2.1G.3.4.1-1. The details of the V2X reference measurement channels (RMCs) are specified in Annexes A.8.3 and reference GNSS configuration as defined in TS 36.508 [7] subclause 4.11.

Table 6.5.2.1G.3.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |
| Test CC Combination setting(NRB_agg) as specified in subclause 5.4.2G |  |  | Lowest NRB_agg, Highest NRB_agg |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| V2X Multi-carrier Configuration |  | V2X PSSCH Allocation |  |  |
| CC1 NRB | CC2 NRB | CC1 & CC2 Mod |  | CC1 and CC2 RB Allocation |
| 50 | 50 | QPSK |  | 48@2+48@2 |
| 50 | 50 | 16QAM |  | 48@2+48@2 |
| Note 1: Only one link has an upload RB allocation when testing EVM. |  |  |  |  |

Table 6.5.2.1G.3.4.1-2: Test Configuration Table for PSBCH

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  |  | Low range, Mid range , High range |  |
| Test Channel Bandwidths as specified in subclause 5.4.2G |  |  |  | Lowest NRB_agg, Highest NRB_agg |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
| V2X multi-carrier Configuration |  |  | V2X PSBCH Configuration |  |  |
| CC1 NRB |  | CC2 NRB | CC1 & CC2 Mod | CC1 and CC2 RB Allocation |  |
| 10MHz |  | 10MHz | QPSK | 6+6 |  |
| Note 1: Only one link has an upload RB allocation when testing EVM. |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors and connect the GNSS simulator to the UE GNSS RX antenna connector as shown in TS 36.508 [7] Annex A, in Figure A.89a.

2.	The parameter setting for the V2X sidelink transmission over PC5 is pre-configured according to TS 36.508[7] subclause 4.10.1.Message content exceptions are defined in clause 6.5.2.1G.3.4.3.

3.	The V2X Reference Measurement Channels are set according to Table 6.5.2.1G.3.4.1-1

4.	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS36.508 [7] Table 4.11.2-2. Geographical area #1 is also pre-configured in the UE.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 5A-V2X in Transmit Mode according to TS 36.508 [7] clause 4.5.9.

7.	The GNSS simulator is triggered to start step 1 of Scenario #1 to simulate a location in the centre of Geographical area #1. Wait for the UE to acquire the GNSS signal and start to transmit.

6.5.2.1G.3.4.2	Test procedure

Test procedure for PSSCH and PSCCH:

1.1	The UE starts to perform the V2X sidelink communication according to SL-V2X-Preconfiguration. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the V2X RMC.

1.2	Configure the UE to transmit at PUMAX level.

1.3	Measure the EVM using Global In-Channel Tx-Test (Annex E) for each component carrier. The measurement period is 15 subframes. When V2X transmissions are shortened due to transmission gap of 1 symbol at the end of the subframe, the EVM measurement interval is reduced by one symbol, accordingly.

1.4	Modify SL-V2X-Preconfiguration to ensure the UE to transmit at a relative low power, according to Table 6.5.2.1G.3.4.3-2.

1.5	Ensure the UE is in State 5A-V2X in Transmit Mode according to TS 36.508 [7] clause 4.5.9 using the new UL power control setting.

1.6	The UE starts to perform the V2X sidelink communication according to SL-V2X-Preconfiguration. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the V2X RMC.

1.7	Measure the EVM using Global In-Channel Tx-Test (Annex E) for each component carrier. The measurement period is 15 subframes. When V2X transmissions are shortened due to transmission gap of 1 symbol at the end of the subframe, the EVM measurement interval is reduced by one symbol, accordingly.

Test procedure for PSBCH:

2.1	The UE starts to perform the V2X PSBCH/SLSS transmission according to SL-V2X-Preconfiguration every 40 subframes. The synchronization configuration of UE is according to Table 6.5.2.1G.3.4.3-3.

2.2	Configure the UE to transmit at PUMAX level.

2.3	Measure the EVM using Global In-Channel Tx-Test (Annex E) until the SS collect 24 subframes of PSBCH for each component carrier. When V2X transmissions are shortened due to transmission gap of 1 symbol at the end of the subframe, the EVM measurement interval is reduced by one symbol, accordingly.

2.4	Modify SL-V2X-Preconfiguration to ensure the UE to transmit at a relative low power, according to Table 6.5.2.1G.3.4.3-3 and Table 6.5.2.1G.3.4.3-4.

2.5	Ensure the UE is in State 5A-V2X in Transmit Mode according to TS 36.508 [7] clause 4.5.9 using the new UL power control setting.

2.6	The UE starts to perform the V2X PSBCH/SLSS transmission according to SL-V2X-Preconfiguration every 40 subframes.

2.7	Measure the EVM using Global In-Channel Tx-Test (Annex E) until the SS collect 24 subframes of PSBCH for each component carrier. When V2X transmissions are shortened due to transmission gap of 1 symbol at the end of the subframe, the EVM measurement interval is reduced by one symbol, accordingly.

6.5.2.1G.3.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6, 4.7.I and 4.10 with the following exceptions:

Table 6.5.2.1G.3.4.3-1: SL-V2X-PreconfigCommPool-r14 configuration for CC1 & CC2 PSSCH/PSCCH test point 1 and 2

| Derivation Path: 36.508 [7] Table 4.6.3-20J |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-V2X-PreconfigCommPool-r14-DEFAULT ::= SEQUENCE { |  |  |  |
| sizeSubchannel-r14 | n50 |  | NRB_alloc = 48, |
| numSubchannel-r14 | n1 |  |  |
| syncAllowed-r14 ::= SEQUENCE { |  |  |  |
| gnss-Sync-r14 | True | The resource of CC1 and CC2 can be used if the UE is directly or indirectly synchronized to GNSS |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.2.1G.3.4.3-2: maxTxPower configuration for CC1 & CC2 PSSCH/PSCCH test point 2

| Derivation Path: 36.508 [7] Table 4.6.3-20K |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-V2X-PreconfigFreqInfo-r14-DEFAULT ::= SEQUENCE { |  |  |  |
| v2x-CommPreconfigGeneral-r14 SEQUENCE { |  |  |  |
| maxTxPower-r12 | -30 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.2.1G.3.4.3-3: synConfig configuration for CC1 & CC2 PSBCH test point 1 and 2

| Derivation Path: 36.508 [7] Table 4.6.3-20K |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-V2X-PreconfigFreqInfo-r14-DEFAULT ::= SEQUENCE { |  |  |  |
| v2x-CommPreconfigSync-r14 SEQUENCE { |  |  |  |
| syncOffsetIndicators-r14 SEQUENCE { |  |  |  |
| syncOffsetIndicator1-r14 | 0 |  |  |
| syncOffsetIndicator2-r14 | 50 |  |  |
| syncOffsetIndicator3-r14 | Not present |  |  |
| } |  |  |  |
| syncTxParameters-r14 | 31dBm |  |  |
| syncTxThreshOoC-r14 | 4 | -90dBm |  |
| filterCoefficient-r14 | fc2 |  |  |
| syncRefMinHyst-r14 | dB6 |  |  |
| syncRefDiffHyst-r14 | dB6 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.2.1G.3.4.3-4: maxTxPower configuration for CC1 & CC2 PSBCH test point 2

| Derivation Path: 36.508 [7] Table 4.6.3-20K |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-V2X-PreconfigFreqInfo-r14-DEFAULT ::= SEQUENCE { |  |  |  |
| v2x-CommPreconfigGeneral-r14 SEQUENCE { |  |  |  |
| maxTxPower-r12 | -30 |  |  |
| } |  |  |  |
| } |  |  |  |

###### 6.5.2.1G.3.5	Test requirement

The PSSCH EVM derived in E.9.1 shall not exceed 17.5% for QPSK, 12.5% for 16 QAM.

The PSCCH EVM and derived in E.9.1 shall not exceed 17.5%.

The PSBCH EVM derived in E.9.2 shall not exceed 17.5%.

#### 6.5.2.1_s	Error Vector Magnitude for subslot/slot TTI

6.5.2.1_s.1	Test purpose

The Error Vector Magnitude is a measure of the difference between the reference waveform and the measured waveform. This difference is called the error vector. Before calculating the EVM the measured waveform is corrected by the sample timing offset and RF frequency offset. Then the carrier leakage shall be removed from the measured waveform before calculating the EVM.

The measured waveform is further modified by selecting the absolute phase and absolute amplitude of the Tx chain. The EVM result is defined after the front-end IDFT as the square root of the ratio of the mean error vector power to the mean reference power expressed as a %.

The basic EVM measurement interval in the time domain is one preamble sequence for the PRACH, and as specified in Table 6.5.2.1_s-1 for the PUCCH and PUSCH in the time domain. When the PUSCH or PUCCH transmission slot or subslot is shortened due to multiplexing with SRS, the EVM measurement interval is reduced by one symbol, accordingly. Likewise, when the PUSCH starting position is modified or when second last symbol is the ending symbol of the PUSCH subframe for Frame Structure Type 3, the EVM measurement interval is reduced accordingly. The PUSCH or PUCCH EVM measurement interval is also reduced when the mean power, modulation or allocation between slots or subslots is expected to change. In the case of PUSCH transmission, the measurement interval is reduced by a time interval equal to the sum of 5 μs and the applicable exclusion period defined in subclause 6.3.4, adjacent to the boundary where the power change is expected to occur. The PUSCH exclusion period is applied to the signal obtained after the front-end IDFT. In the case of PUCCH transmission with power change, the PUCCH EVM measurement interval is reduced by one symbol adjacent to the boundary where the power change is expected to occur.

Table 6.5.2.1_s-1: Measurement interval for EVM

| TTI pattern | Measurement interval |
| --- | --- |
| Subframe | 7OS |
| Slot | 7OS |
| Subslot | 2OS, 3OS |

6.5.2.1_s.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward that support subslot/slot TTI.

6.5.2.1_s.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.2.1.3.

6.5.2.1_s.4	Test description

6.5.2.1_s.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.5.2.1_s.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.2.1_s.4.1-1: Test Configuration Table for PUSCH

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  | NC |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  | See Table 6.5.1.4.1-1 |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  | See Table 6.5.1.4.1-1 |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
|  | Downlink Configuration | Uplink Configuration |  |  |
| Ch BW | N/A for PUSCH EVM testing | Modulation | RB allocation |  |
|  |  |  | FDD | TDD |
| 1.4MHz |  | QPSK | 6 | 6 |
| 1.4MHz |  | QPSK | 1 | 1 |
| 1.4MHz |  | 16QAM | 6 | 6 |
| 1.4MHz |  | 16QAM | 1 | 1 |
| 3MHz |  | QPSK | 15 | 15 |
| 3MHz |  | QPSK | 4 | 4 |
| 3MHz |  | 16QAM | 15 | 15 |
| 3MHz |  | 16QAM | 4 | 4 |
| 5MHz |  | QPSK | 25 | 25 |
| 5MHz |  | QPSK | 8 | 8 |
| 5MHz |  | 16QAM | 25 | 25 |
| 5MHz |  | 16QAM | 8 | 8 |
| 10MHz |  | QPSK | 50 | 50 |
| 10MHz |  | QPSK | 12 | 12 |
| 10MHz |  | 16QAM | 50 (Note 3) | 50 (Note 3) |
| 10MHz |  | 16QAM | 12 | 12 |
| 15MHz |  | QPSK | 75 | 75 |
| 15MHz |  | QPSK | 16 | 16 |
| 15MHz |  | 16QAM | 75 (Note 3) | 75 (Note 3) |
| 15MHz |  | 16QAM | 16 | 16 |
| 20MHz |  | QPSK | 100 | 100 |
| 20MHz |  | QPSK | 18 | 18 |
| 20MHz |  | 16QAM | 100 (Note 3) | 100 (Note 3) |
| 20MHz |  | 16QAM | 18 | 18 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: For partial RB allocation, the RBstart shall be RB #0 and RB# (max+1 - RB allocation) of the channel bandwidth. Note 3: Applies only for UE-Categories ≥2 |  |  |  |  |

Table 6.5.2.1_s.4.1-2: Test Configuration Table for PUCCH

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  | NC |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 |  |  | See Table 6.5.1.4.1-1 |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | See Table 6.5.1.4.1-1 |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |
| Ch BW | Mod'n | RB allocation |  | FDD: PUCCH format = Format 1a TDD: PUCCH format = Format 1a / 1b |
|  |  | FDD | TDD |  |
| 1.4MHz | QPSK | 3 | 3 |  |
| 3MHz | QPSK | 4 | 4 |  |
| 5MHz | QPSK | 8 | 8 |  |
| 10MHz | QPSK | 16 | 16 |  |
| 15MHz | QPSK | 25 | 25 |  |
| 20MHz | QPSK | 30 | 30 |  |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. |  |  |  |  |

Table 6.5.2.1_s.4.1-3: Test Configuration for PRACH

| Initial Conditions |  |  |
| --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) | NC |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) | See Table 6.5.1.4.1-1 |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) | See Table 6.5.1.4.1-1 |  |
| PRACH preamble format |  |  |
|  | FDD | TDD |
| PRACH Configuration Index | 4 | 53 |
| RS EPRE setting for test point 1 (dBm/15kHz) | -71 | -63 |
| RS EPRE setting for test point 2 (dBm/15kHz) | -86 | -78 |

1.	Connect the SS to the UE to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.3.

2.	The parameter settings for the cell are set up according to TS 36.508[7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to in Table 6.5.2.1_s.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.5.2.1_s.4.3.

6.5.2.1_s.4.2	Test procedure

Test procedure for subslot/slot PUSCH:

1.1.	SS sends uplink scheduling information for each UL HARQ process via SPDCCH DCI format 7-0A for C_RNTI to schedule the UL RMC according to Table 6.5.2.1_s.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

1.2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

1.3.	Measure the EVM and  using Global In-Channel Tx-Test (Annex E).

1.4.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is –36.8dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -36.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

1.5.	Measure the EVM and  using Global In-Channel Tx-Test (Annex E).

Test procedure for subslot/slot PUCCH:

2.1.	PUCCH are set according to Table 6.5.2.1_s.4.1-2.

2.2.	SS transmits PDSCH via PDCCH DCI format 1A for C_RNTI to transmit the DL RMC according to Table 6.5.2.1_s.4.1-2. The SS sends downlink MAC padding bits on the DL RMC. The transmission of PDSCH will make the UE send uplink ACK/NACK using subslot/slot PUCCH. There is no PUSCH transmission.

2.3.	SS send appropriate TPC commands for subslot/slot PUCCH to the UE until the UE transmit subslot/slot PUCCH at PUMAX level.

2.4.	Measure subslot/slot PUCCH EVM using Global In-Channel Tx-Test (Annex E).

2.5.	Send the appropriate TPC commands for subslot/slot PUCCH to the UE until the UE transmits subslot/slot PUCCH at -36.8dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -36.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

2.6.	Measure subslot/slot PUCCH EVM using Global In-Channel Tx-Test (Annex E).

Test procedure for PRACH:

3.1.	The SS shall set RS EPRE according to Table 6.5.2.1_s.4.1-3.

3.2.	PRACH is set according to Table 6.5.2.1_s.4.1-3.

3.3.	The SS shall signal a Random Access Preamble ID via a PDCCH order to the UE and initiate a Non-contention based Random Access procedure

3.4.	The UE shall send the signalled preamble to the SS.

3.5.	In response to the preamble, the SS shall transmit a random access response not corresponding to the transmitted random access preamble, or send no response.

3.6.	The UE shall consider the random access response reception not successful then re-transmit the preamble with the calculated PRACH transmission power.

3.7.	Repeat step 5 and 6 until the SS collect enough PRACH preambles (2 preambles for format 0 and 10 preambles for format 4). Measure the EVM in PRACH channel using Global In-Channel Tx-Test (Annex E).

6.5.2.1_s.4.3	Message contents

Same message contents as in clause 6.5.2.1.4.3 with the following exceptions:

Table 6.5.2.1_s.4.3-5: PhysicalConfigDedicated-DEFAULT (preamble)

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |  |
| PhysicalConfigDedicatedSTTI-r15 SEQUENCE { |  |  |  |  |
| shortTTI-r15 SEQUENCE { |  |  |  |  |
| dl-STTI-Length-r15 |  | subslot |  | FDD_sTTI-22 OR FDD_sTTI-27 |
| ul-STTI-Length-r15 |  | subslot |  | FDD_sTTI-22 |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD_sTTI-22 | FDD cell environment and UEs supporting {subslot, subslot} combination |
| FDD_sTTI-27 | FDD cell environment and UEs supporting {subslot, slot} combination |

6.5.2.1_s.5	Test requirements

Same test requirements as in clause 6.5.2.1.5.

#### 6.5.2.2	Carrier leakage

##### 6.5.2.2.1	Test Purpose

Carrier leakage expresses itself as unmodulated sine wave with the carrier frequency or centre frequency of aggregated transmission bandwidth configuration. It is an interference of approximately constant amplitude and independent of the amplitude of the wanted signal. Carrier leakage interferes with the centre sub carriers of the UE under test (if allocated), especially, when their amplitude is small. The measurement interval is defined over one slot in the time domain.

The purpose of this test is to exercise the UE transmitter to verify its modulation quality in terms of carrier leakage.

##### 6.5.2.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 8 and forward.

##### 6.5.2.2.3	Minimum conformance requirements

The relative carrier leakage power is a power ratio of the additive sinusoid waveform and the modulated waveform. The relative carrier leakage power shall not exceed the values specified in Table 6.5.2.2.3-1 or 6.5.2.2.3-2.

Table 6.5.2.2.3-1: Minimum requirements for Relative Carrier Leakage Power for UE supporting Rel-8 to Rel-10

| LO Leakage | Parameters | Relative Limit (dBc) |
| --- | --- | --- |
|  | Output power >0 dBm | -25 |
|  | -30 dBm ≤ Output power ≤0 dBm | -20 |
|  | -40 dBm Output power < -30 dBm | -10 |

Table 6.5.2.2.3-2: Minimum requirements for Relative Carrier Leakage Power for UE supporting Rel.11 and higher

| Parameters | Relative limit (dBc) | Applicable frequencies |
| --- | --- | --- |
| Output power >10 dBm | -28 | Carrier centre frequency < 1 GHz |
|  | -25 | Carrier centre frequency ≥ 1 GHz |
| 0 dBm ≤ Output power ≤10 dBm | -25 |  |
| -30 dBm ≤ Output power ≤0 dBm | -20 |  |
| -40 dBm Output power < -30 dBm | -10 |  |

The normative reference for this requirement is TS 36.101 clause 6.5.2.2.1

##### 6.5.2.2.4	Test description

6.5.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.5.2.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.2.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  | See Table 6.5.1.4.1-1 |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  | See Table 6.5.1.4.1-1 |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  | See Table 6.5.1.4.1-1 |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
|  | Downlink Configuration | Uplink Configuration |  |  |
| Ch BW | N/A for carrier leakage testing | Mod'n | RB allocation |  |
|  |  |  | FDD | TDD |
| 1.4MHz |  | QPSK | 1 | 1 |
| 3MHz |  | QPSK | 4 | 4 |
| 5MHz |  | QPSK | 8 | 8 |
| 10MHz |  | QPSK | 12 | 12 |
| 15MHz |  | QPSK | 16 | 16 |
| 20MHz |  | QPSK | 18 | 18 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: For partial RB allocation, the RBstart shall be RB #0 and RB# (max +1- RB allocation) of the channel bandwidth. |  |  |  |  |

1.	Connect the SS to the UE to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.3.

2.	The parameter settings for the cell are set up according to TS 36.508[7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to in Table 6.5.2.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.5.2.2.4.3.

6.5.2.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.5.2.2.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC

2.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is 13.2 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or 13.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

3.	Measure carrier leakage using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test.

4.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is 3.2 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or 3.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

5.	Measure carrier leakage using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test.

6.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is -26.8 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -26.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

7.	Measure carrier leakage using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test

8.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is -36.8dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -36.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

9.	Measure carrier leakage using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test

NOTE:	The step 2 and 3 only apply to UE supporting Rel.11 and higher.

6.5.2.2.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6.

##### 6.5.2.2.5	Test requirement

Each of the 20 carrier leakage results, derived in Annex E.3.1, shall not exceed the values in table 6.5.2.2.5-1 or 6.5.2.2.5-2.

Table 6.5.2.2.5-1: Test requirements for Relative Carrier Leakage Power for UE supporting Rel-8 to Rel-10

| LO Leakage | Parameters | Relative Limit (dBc) |
| --- | --- | --- |
|  | f ≤ 3.0GHz: 3.2 dBm ±3.2dB 3.0GHz < f ≤ 4.2GHz: 3.5 dBm ±3.5dB | -24.2 |
|  | f ≤ 3.0GHz: -26.8 dBm ±3.2dB 3.0GHz < f ≤ 4.2GHz: -26.5 dBm ±3.5dB | -19.2 |
|  | f ≤ 3.0GHz: -36.8dBm±3.2dB 3.0GHz < f ≤ 4.2GHz: -36.5 dBm ±3.5dB | -9.2 |

Table 6.5.2.2.5-2: Test requirements for Relative Carrier Leakage Power for UE supporting Rel.11 and higher

| LO Leakage | Parameters | Relative limit (dBc) | Applicable frequencies |
| --- | --- | --- | --- |
|  | f ≤ 3.0GHz: 13.2 dBm ±3.2dB 3.0GHz < f ≤ 4.2GHz: 13.5 dBm ±3.5dB | -27.2 | Carrier centre frequency < 1 GHz |
|  |  | -24.2 | Carrier centre frequency ≥ 1 GHz |
|  | f ≤ 3.0GHz: 3.2 dBm ±3.2dB 3.0GHz < f ≤ 4.2GHz: 3.5 dBm ±3.5dB | -24.2 |  |
|  | f ≤ 3.0GHz: -26.8 dBm ±3.2dB 3.0GHz < f ≤ 4.2GHz: -26.5 dBm ±3.5dB | -19.2 |  |
|  | f ≤ 3.0GHz: -36.8dBm±3.2dB 3.0GHz < f ≤ 4.2GHz: -36.5 dBm ±3.5dB | -9.2 |  |

#### 6.5.2.2E	Carrier leakage for UE category 0

##### 6.5.2.2E.1	Test Purpose

Same test purpose in clause 6.5.2.2.1.

##### 6.5.2.2E.2	Test applicability

This test case applies to all types of E-UTRA UE release 12 and forward of UE category 0.

##### 6.5.2.2E.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3.2.3.

##### 6.5.2.2E.4	Test description

6.5.2.2E.4.1	Initial conditions

Same initial conditions as in clause 6.5.2.2.4.1 with the following exceptions:

-	Instead of Table 6.5.2.2.4.1-1  use Table 6.5.2.2E.4.1-1.

-	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.3 using only main UE Tx/Rx antenna.

Table 6.5.2.2E.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  | See Table 6.5.1.4.1-1 |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  | See Table 6.5.1.4.1-1 |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  | See Table 6.5.1.4.1-1 |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
|  | Downlink Configuration | Uplink Configuration |  |  |
| Ch BW | N/A for carrier leakage testing | Mod'n | RB allocation |  |
|  |  |  | FDD and HD-FDD | TDD |
| 1.4MHz |  | QPSK | 1 | 1 |
| 3MHz |  | QPSK | 4 | 4 |
| 5MHz |  | QPSK | 8 | 8 |
| 10MHz |  | QPSK | 12 | 12 |
| 15MHz |  | QPSK | 16 | 16 |
| 20MHz |  | QPSK | 18 | 18 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: For partial RB allocation, the RBstart shall be RB #0 and RB# (max +1- RB allocation) of the channel bandwidth. |  |  |  |  |

6.5.2.2.4.2	Test procedure

Same test procedure as in clause 6.5.2.2.4.1 with the following exception for HD-FDD:

-	Instead of Table 6.5.2.2.4.1-1  use Table 6.5.2.2E.4.1-1.

-	In step 3, slots with transient periods are not under test. Half-duplex guard subframes are not under test.

6.5.2.2E.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6.

##### 6.5.2.2E.5	Test requirement

Each of the 20 IQ offset results, derived in Annex E.3.1, shall not exceed the values in table 6.5.2.2E.5-1

Table 6.5.2.2E.5-1: Test requirements for Relative Carrier Leakage Power

| LO Leakage | Parameters | Relative Limit (dBc) |
| --- | --- | --- |
|  | f ≤ 3.0GHz: 3.2 dBm ±3.2dB 3.0GHz < f ≤ 4.2GHz: 3.5 dBm ±3.5dB | -24.2 |
|  | f ≤ 3.0GHz: -26.8 dBm ±3.2dB 3.0GHz < f ≤ 4.2GHz: -26.5 dBm ±3.5dB | -19.2 |
|  | f ≤ 3.0GHz: -36.8dBm±3.2dB 3.0GHz < f ≤ 4.2GHz: -36.5 dBm ±3.5dB | -9.2 |

#### 6.5.2.2EA	Carrier leakage for UE category M1

##### 6.5.2.2EA.1	Test Purpose

Same test purpose in clause 6.5.2.2.1.

##### 6.5.2.2EA.2	Test applicability

This test case applies to all types of E-UTRA UE release 13 and forward of UE category M1.

##### 6.5.2.2EA.3	Minimum conformance requirements

Carrier leakage is an additive sinusoid waveform that has the same frequency as a modulated waveform carrier frequency. For UE of UL Category M1, the sinusoid waveform may alternatively lie at the centre of the 6 RB narrowband assigned for transmission. The measurement interval is one slot in the time domain.

The relative carrier leakage power is a power ratio of the additive sinusoid waveform and the modulated waveform. The relative carrier leakage power at the centre of the channel bandwidth or the 6 RB narrowband assigned for transmission shall not exceed the values specified in Table 6.5.2.2.3-2. The normative reference for this requirement is TS 36.101 [2] clause 6.5.2E.2.1.

##### 6.5.2.2EA.4	Test description

6.5.2.2EA.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in sub-clause 5.2E. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.5.2.2EA.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and MPDCCH before measurement are specified in Annex C.2.

Table 6.5.2.2EA.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  | See Table 6.5.1.4.1-1 |  |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  | See Table 6.5.1.4.1-1 |  |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  | 5MHz |  |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  | Downlink Configuration | Uplink Configuration |  |  |  |
| Ch BW | N/A for carrier leakage testing | Mod'n | RB allocation |  |  |
|  |  |  | FDD and HD-FDD | TDD | Narrowband index1 |
| 5MHz |  | QPSK | 1 | 1 | 0 |
| 5MHz |  | QPSK | 6 | 6 | 0 |
| 5MHz |  | π/2-BPSK | ¼ (Note 3) | ¼ (Note 3) | 0 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4 Note 2: For partial RB allocation, the RBstart shall be RB #0 and RB# (6 - RB allocation) of the channel bandwidth. Note 3: Only applicable for UE supporting subPRB allocation |  |  |  |  |  |

1.	Connect the SS to the UE to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508[7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to in Table 6.5.2.2EA.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [7] clause 5.2A.2AA. Message contents are defined in clause 6.5.2.2EA.4.3.

6.5.2.2EA.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via MPDCCH DCI format 6-0A for C_RNTI to schedule the UL RMC according to Table 6.5.2.2EA.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC

2.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is 13.2 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or 13.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

3.	Measure carrier leakage using Global In-Channel Tx-Test (Annex E) according to the UE’s declaration on the position of carrier centre frequency. For TDD slots with transient periods are not under test. For HD-FDD slots with transient periods are not under test. Half-duplex guard subframes are not under test.

4.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is 3.2 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or 3.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

5.	Measure carrier leakage using Global In-Channel Tx-Test (Annex E) according to the UE’s declaration on the position of carrier centre frequency. For TDD slots with transient periods are not under test. For HD-FDD slots with transient periods are not under test. Half-duplex guard subframes are not under test.

6.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is -26.8 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -26.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

7.	Measure carrier leakage using Global In-Channel Tx-Test (Annex E) according to the UE’s declaration on the position of carrier centre frequency. For TDD slots with transient periods are not under test. For HD-FDD slots with transient periods are not under test. Half-duplex guard subframes are not under test.

8.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is -36.8dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -36.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

9.	Measure carrier leakage using Global In-Channel Tx-Test (Annex E) according to the UE’s declaration on the position of carrier centre frequency. For TDD slots with transient periods are not under test

6.5.2.2EA.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the condition CEModeA.

##### 6.5.2.2EA.5	Test requirement

Each of the 20 IQ offset results, derived in Annex E.3.1, shall not exceed the values in table 6.5.2.2EA.5-1

Table 6.5.2.2EA.5-1: Test requirements for Relative Carrier Leakage Power

| LO Leakage | Parameters | Relative limit (dBc) | Applicable frequencies |
| --- | --- | --- | --- |
|  | f ≤ 3.0GHz: 13.2 dBm ±3.2dB 3.0GHz < f ≤ 4.2GHz: 13.5 dBm ±3.5dB | -27.2 | Carrier centre frequency < 1 GHz |
|  |  | -24.2 | Carrier centre frequency ≥ 1 GHz |
|  | f ≤ 3.0GHz: 3.2 dBm ±3.2dB 3.0GHz < f ≤ 4.2GHz: 3.5 dBm ±3.5dB | -24.2 |  |
|  | f ≤ 3.0GHz: -26.8 dBm ±3.2dB 3.0GHz < f ≤ 4.2GHz: -26.5 dBm ±3.5dB | -19.2 |  |
|  | f ≤ 3.0GHz: -36.8dBm±3.2dB 3.0GHz < f ≤ 4.2GHz: -36.5 dBm ±3.5dB | -9.2 |  |

#### 6.5.2.2EB	Carrier leakage for UE category 1bis

##### 6.5.2.2EB.1	Test purpose

Same test purpose as in clause 6.5.2.2.

##### 6.5.2.2EB.2	Test applicability

This test applies to all types of E-UTRA UE release 13 and forward of UE category 1bis.

##### 6.5.2.2EB.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.2.2.3.

##### 6.5.2.2EB.4	Test description

6.5.2.2EB.4.1	Initial conditions

Same initial conditions as in clause 6.5.2.2.4.1 with the following exception:

-	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.3 using only main UE Tx/Rx antenna.

6.5.2.2EB.4.2	Test procedure

Same test procedure as in clause 6.5.2.2.4.2.

6.5.2.2EB.4.3	Message contents

Same message contents as in clause 6.5.2.2.4.3.

##### 6.5.2.2EB.5	Test requirement

Each of the 20 carrier leakage results, derived in Annex E.3.1, shall not exceed the values in Table 6.5.2.2EB.5-1.

Table 6.5.2.2EB.5-1: Test requirements for Relative Carrier Leakage Power for UE supporting Rel.13 and higher

| LO Leakage | Parameters | Relative limit (dBc) | Applicable frequencies |
| --- | --- | --- | --- |
|  | f ≤ 3.0GHz: 13.2 dBm ±3.2dB 3.0GHz < f ≤ 4.2GHz: 13.5 dBm ±3.5dB | -27.2 | Carrier centre frequency < 1 GHz |
|  |  | -24.2 | Carrier centre frequency ≥ 1 GHz |
|  | f ≤ 3.0GHz: 3.2 dBm ±3.2dB 3.0GHz < f ≤ 4.2GHz: 3.5 dBm ±3.5dB | -24.2 |  |
|  | f ≤ 3.0GHz: -26.8 dBm ±3.2dB 3.0GHz < f ≤ 4.2GHz: -26.5 dBm ±3.5dB | -19.2 |  |
|  | f ≤ 3.0GHz: -36.8dBm±3.2dB 3.0GHz < f ≤ 4.2GHz: -36.5 dBm ±3.5dB | -9.2 |  |

#### 6.5.2.2EC	Carrier leakage for UE category M2

##### 6.5.2.2EC.1	Test Purpose

Same test purpose in clause 6.5.2.2.1.

##### 6.5.2.2EC.2	Test applicability

This test case applies to all types of E-UTRA UE release 14 and forward of UE category M2.

##### 6.5.2.2EC.3	Minimum conformance requirements

Carrier leakage is an additive sinusoid waveform that has the same frequency as a modulated waveform carrier frequency. For UE of UL Categories M1 and M2, the sinusoid waveform may lie at the centre of the narrowband assigned for transmission. The measurement interval is one slot in the time domain.

The relative carrier leakage power is a power ratio of the additive sinusoid waveform and the modulated waveform. The relative carrier leakage power at the centre of the channel bandwidth or the centre of the narrowband assigned for transmission shall not exceed the values specified in Table 6.5.2.2.3-2.

The normative reference for this requirement is TS 36.101 [2] clause 6.5.2E.2.

##### 6.5.2.2EC.4	Test description

6.5.2.2EC.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in sub-clause 5.2E. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.5.2.2EC.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and MPDCCH before measurement are specified in Annex C.2.

Table 6.5.2.2EC.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  | Low range, Mid range, High range |  |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  | 5MHz, Highest |  |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  | Downlink Configuration | Uplink Configuration |  |  |  |
| Ch BW | N/A for carrier leakage testing | Mod'n | RB allocation |  |  |
|  |  |  | FDD and HD-FDD | TDD | Narrowband index1 |
| Low range, Mid range |  |  |  |  |  |
| 5MHz |  | QPSK | 1 | 1 | 0 |
| 5MHz |  | QPSK | 24 | 24 | 0 |
| 10MHz |  | QPSK | 1 | 1 | 0 |
| 10MHz |  | QPSK | 24 | 24 | 0 |
| 15MHz |  | QPSK | 1 | 1 | 0 |
| 15MHz |  | QPSK | 24 | 24 | 0 |
| 20MHz |  | QPSK | 1 | 1 | 0 |
| 20MHz |  | QPSK | 24 | 24 | 0 |
| 5MHz |  | π/2-BPSK | ¼3 | ¼3 | 0 |
| 10MHz |  | π/2-BPSK | ¼3 | ¼3 | 0 |
| 15MHz |  | π/2-BPSK | ¼3 | ¼3 | 0 |
| 20MHz |  | π/2-BPSK | ¼3 | ¼3 | 0 |
| High range |  |  |  |  |  |
| 5MHz |  | QPSK | 1 | 1 | 3 |
| 5MHz |  | QPSK | 24 | 24 | 0 |
| 10MHz |  | QPSK | 1 | 1 | 7 |
| 10MHz |  | QPSK | 24 | 24 | 4 |
| 15MHz |  | QPSK | 1 | 1 | 11 |
| 15MHz |  | QPSK | 24 | 24 | 8 |
| 20MHz |  | QPSK | 1 | 1 | 15 |
| 20MHz |  | QPSK | 24 | 24 | 12 |
| 5MHz |  | π/2-BPSK | ¼3 | ¼3 | 0 |
| 10MHz |  | π/2-BPSK | ¼3 | ¼3 | 0 |
| 15MHz |  | π/2-BPSK | ¼3 | ¼3 | 0 |
| 20MHz |  | π/2-BPSK | ¼3 | ¼3 | 0 |
| Note 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.7. Note 2: The RBstart of the 1RB allocation shall be RB#0 of the narrowband for Low/Mid frequency range and RB#5 of the narrowband for High frequency range. Note 3: Only applicable for UE supporting subPRB allocation. |  |  |  |  |  |

1.	Connect the SS to the UE to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508[7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to in Table 6.5.2.2EC.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [7] clause 5.2A.2AA. Message contents are defined in clause 6.5.2.2EC.4.3.

6.5.2.2EC.4.2	Test procedure

Same test procedure as in clause 6.5.2.2EA.4.2 with the following exception.

-	Instead of Table 6.5.2.2EA.4.1-1  use Table 6.5.2.2EC.4.1-1.

6.5.2.2EC.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the condition CEModeA.

6.5.2.2EC.5	Test requirement

Same test requirement as in clause 6.5.2.2EA.5.

#### 6.5.2.2F	Carrier leakage for category NB1 and NB2

6.5.2.2F.1	Test Purpose

Carrier leakage expresses itself as unmodulated sine wave with the carrier frequency or centre frequency of aggregated transmission bandwidth configuration. It is an interference of approximately constant amplitude and independent of the amplitude of the wanted signal. Carrier leakage interferes with the centre sub carriers of the UE under test (if allocated), especially, when their amplitude is small. The measurement interval is defined over one slot in the time domain.

The purpose of this test is to exercise the UE transmitter to verify its modulation quality in terms of carrier leakage.

6.5.2.2F.2	Test applicability

This test case applies to all types of E-UTRA FDD UE release 13 and forward of category NB1.

This test case applies to all types of E-UTRA FDD UE release 14 and forward of category NB2.

This test case applies to all types of E-UTRA TDD UE release 15 and forward of category NB1 and NB2.

6.5.2.2F.3	Minimum conformance requirements

Carrier leakage is an additive sinusoid waveform that has the same frequency as a modulated waveform carrier frequency. The measurement interval is one slot in the time domain. The relative carrier leakage power is a power ratio of the additive sinusoid waveform and the modulated waveform. The relative carrier leakage power of category NB1 and NB2 UE shall not exceed the values specified in Table 6.5.2.2F.3-1.

Table 6.5.2.2F.3-1: Minimum requirements for relative carrier leakage power

| Parameters | Relative limit (dBc) |
| --- | --- |
| 0 dBm ≤ Output power | -25 |
| -30 dBm ≤ Output power ≤ 0 dBm | -20 |
| -40 dBm Output power < -30 dBm | -10 |

The normative reference for this requirement is TS 36.101 clause 6.5.2F.2

##### 6.5.2.2F.4	Test description

6.5.2.2F.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions and test frequencies based on the subset of E-UTRA operating bands defined for NB-IoT in clause 5.2F. All of these configurations in table 6.5.2.2F.4.1-1 shall be tested. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2.4. Configurations of NPDSCH and NPDCCH before measurement are specified in Annex TBD.

Table 6.5.2.2F.4.1-1: Test Configuration Table for FDD & TDD

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 8.1.1 |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 8.1.3.1 |  | Frequency ranges defined in Annex K.1.1 |  |  |
| Test Parameters |  |  |  |  |
| Configuration ID | Downlink Configuration | Uplink Configuration |  |  |
|  | N/A | Modulation | Ntones, start position | Sub-carrier spacing (kHz) |
| 1 |  | QPSK | 1@0 | 3.75 |
| 2 |  | QPSK | 1@47 | 3.75 |
| 3 |  | QPSK | 1@0 | 15 |
| 4 |  | QPSK | 1@11 | 15 |

1.	Connect the SS to the UE to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508[7] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex H.

4.	The UL Reference Measurement channels are set according to in Table 6.5.2.2F.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in 2A-NB with CP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5. Message contents are defined in clause 6.5.2.2F.4.3.

6.5.2.2F.4.2	Test procedure

1.	SS sends uplink scheduling information for UL HARQ process via NPDCCH DCI format N0 for C_RNTI to schedule the UL RMC according to Table 6.5.2.2F.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC

2.	Configure UE to transmit at PUMAX level.

3.	Measure carrier leakage using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test.

4. Release the connection through State 3A-NB.

5. Modify system information elements according to Table 6.5.2.1F.1.4.3-1 and Table 6.5.2.1F.1.4.3-2 and notify the UE via paging message with SystemInformationModification included (test point 2).

6. Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5 using the new UL power control setting.

7. SS sends uplink scheduling information for UL HARQ process via NPDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.5.2.2F.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

8.	Measure carrier leakage using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test.

NOTE 1:	For configuration IDs applicable to UE depending on UE capability in Test Configuration Table with different UL sub-carrier spacing, the SS shall release the connection through State 3A-NB and finally ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5 using the appropriate UL subcarrier spacing in Random Access Response message.

6.5.2.2F.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 8.1.6 with the following exceptions:

Table 6.5.2.1F.1.4.3-1: P0-NominalNPUSCH-r13 configuration for test point 2

| Derivation Path: 36.508 clause 8.1.6.3 Table 8.1.6.3-14: UplinkPowerControlCommon-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| p0-NominalNPUSCH-r13 | -117 (dBm) |  |  |
| alpha-r13 | al1 (1) |  |  |
| deltaPreambleMsg3-r13 | 4 |  |  |
| } |  |  |  |

Table 6.5.2.1F.1.4.3-2: NPDSCH-ConfigCommon-NB-DEFAULT configuration for test point 2

| Derivation Path: 36.508 clause 8.1.6.3 Table 8.1.6.3-4: NPDSCH-ConfigCommon-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPDSCH-ConfigCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| nrs-Power-r13 | 21 (dBm) |  |  |
| } |  |  |  |

##### 6.5.2.2F.5	Test requirement

Each of the carrier leakage results, derived in Annex E.3.1, shall not exceed the values in table 6.5.2.2F.5-1

Table 6.5.2.2F.5-1: Test requirements for Relative Carrier Leakage Power

| LO Leakage | Parameters | Relative Limit (dBc) |
| --- | --- | --- |
|  | Test point 1 | -24.2 |
|  | Test point 2 | -19.2 |

#### 6.5.2.2G	Carrier leakage for V2X Communication

##### 6.5.2.2G.0	Minimum conformance requirements

The requirements of subclause 6.5.2.2.3 shall apply for V2X transmissions.

For intra-band contiguous multi-carrier operation the carrier leakage requirement of 6.5.2A.2.0 shall apply.

When UE is configured for simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions for inter-band E-UTRA V2X / E-UTRA bands specified in Table 5.2G-2, the requirements in subclause 6.5.2.2G.0 apply for V2X sidelink transmission and the requirements in subclause 6.5.2.2.3 apply for the E-UTRA uplink transmission.

The normative reference for this requirement is TS 36.101 [2] clause 6.5.2G and 6.5.2G.2.

##### 6.5.2.2G.1	Carrier leakage for V2X Communication / Non-concurrent with E-UTRA uplink transmissions

###### 6.5.2.2G.1.1	Test Purpose

Same test purpose in clause 6.5.2.2.1.

###### 6.5.2.2G.1.2	Test applicability

This test case applies to all types of UE that supports V2X Sidelink communication and Band 47.

###### 6.5.2.2G.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5.2.2G.0.

###### 6.5.2.2G.1.4	Test description

6.5.2.2G.1.4.1		Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidth based on E-UTRAN operating bands specified in table 5.4.2G.1-2. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.5.2.2G.1.4.1-1. The details of the reference measurement channels (RMCs) are specified in Annex A.8.3 and the GNSS configuration in TS 36.508 [7] subclause 4.11.

Table 6.5.2.2G.1.4.1-1: Test Configuration Table for PSSCH and PSCCH

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |
|  | V2X Configuration to Transmit |  |  |
| Ch BW | Modulation |  | PSSCH RB allocation (Note 1) |
| 10 MHz | QPSK |  | 12 |
| 20 MHz | QPSK |  | 18 |
| Note 1: The test shall be performed with allocated sub-channel(s) at low and high end of the channel bandwidth respectively. |  |  |  |

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 36.508 [7] Figure A.92.

2.	The parameter settings for the V2X sidelink transmission over PC5 for both SS and UE are pre-configured according to TS 36.508 [7] Table 4.10.1.1-1. Message content exceptions are defined in clause 6.5.2.2G.1.4.3.

3.	The V2X reference measurement channel is set according to Table 6.5.2G.1.4.1-1.

4.	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS36.508 [7] Table 4.11.2-2. Geographical area #1 is also pre-configured in the UE.

5.	Propagation conditions are set according to Annex B.0;

6.	Ensure the UE is in State 5A-V2X in Transmit Mode according to TS 36.508 [7] clause 4.5.9.

7.	The GNSS simulator is triggered to start step 1 of Scenario #1 to simulate a location in the centre of Geographical area #1. Wait for the UE to acquire the GNSS signal and start to transmit.

6.5.2.2G.1.4.2	Test procedure

TP 1:

1	Set the V2X sidelink communication pre-configuration parameters for UE according to TS 36.508[7] Table 4.10.1.1-1 with the exception specified in table 6.5.2.2G.3.4.3-1 and Table 6.5.2.2G.3.4.3-2;

2	The UE starts to perform the V2X sidelink communication according to SL-V2X-Preconfiguration. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the V2X RMC.

3	Measure the carrier leakage using Global In-Channel Tx-Test (Annex E).

TP 2:

4	Set the V2X sidelink communication pre-configuration parameters for UE according to TS 36.508[7] Table 4.10.1.1-1 with the exception specified in table 6.5.2.2G.3.4.3-1 and Table 6.5.2.2G.3.4.3-3;

5	The UE starts to perform the V2X sidelink communication according to SL-V2X-Preconfiguration. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the V2X RMC.

6	Measure the carrier leakage using Global In-Channel Tx-Test (Annex E).

6.5.2.2G.1.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6, 4.7.I and 4.10 with the following exceptions:

Table 6.5.2.2G.1.4.3-1: Void

Table 6.5.2.2G.1.4.3-2: maxTxPower configuration for test point 1

| Derivation Path: 36.508 Table 4.6.3-20K |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-V2X-PreconfigFreqInfo-r14-DEFAULT ::= SEQUENCE { |  |  |  |
| v2x-CommPreconfigGeneral-r14 SEQUENCE { |  |  |  |
| maxTxPower-r12 | 15 | Test point 1 checks the requirement for Output power > 0dBm |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.2.2G.1.4.3-3: maxTxPower configuration for test point 2

| Derivation Path: 36.508 Table 4.6.3-20K |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-V2X-PreconfigFreqInfo-r14-DEFAULT ::= SEQUENCE { |  |  |  |
| v2x-CommPreconfigGeneral-r14 SEQUENCE { |  |  |  |
| maxTxPower-r12 | -15 | Test point 2 checks the requirement for -30dBm <= Output power < 0dBm |  |
| } |  |  |  |
| } |  |  |  |

###### 6.5.2.2G.1.5	Test requirement

Each of the 20 carrier leakage results, derived in Annex E.99, shall not exceed the values in table 6.5.2.2G.5-1.

Table 6.5.2.2G.5-1: Test requirements for Relative Carrier Leakage Power

| LO Leakage | Parameters | Relative Limit (dBc) |
| --- | --- | --- |
|  | Normal conditions:15 dBm ±10.5dB Extreme conditions: 15 dBm ±14.5dB | -24.2 |
|  | Normal conditions:-15 dBm ±10.5dB Extreme conditions: -15 dBm ±14.5dB | -19.2 |

##### 6.5.2.2G.2	Carrier leakage for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmission

###### 6.5.2.2G.2.1	Test Purpose

Same test purpose in clause 6.5.2.2.1.

###### 6.5.2.2G.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 14 and forward that support V2X Communication.

###### 6.5.2.2G.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5.2.2G.0.

###### 6.5.2.2G.2.4	Test description

6.5.2.2G.2.4.1		Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidth based on E-UTRA operating bands specified in table 5.4.2G.1-2. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.5.2.2G.2.4.1-1. The details of the uplink and V2X reference measurement channels (RMCs) are specified in Annexes A.2 and A.8.3 respectively. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.2.2G.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range for uplink and Low range for V2X, High range for uplink and High range for V2X |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Lowest NRB_agg, Highest NRB_agg |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  | Downlink | Uplink Configuration |  | V2X Configuration |  |  |
| Ch BW | N/A | Mod'n | RB allocation (Note 2) | Ch BW | Mod'n | RB allocation (Note 4) |
| 1.4 MHz |  | QPSK | 1 | 10 MHz | QPSK | 12 |
| 3 MHz |  | QPSK | 4 | 20 MHz | QPSK | 18 |
| 5 MHz |  | QPSK | 8 |  |  |  |
| 10 MHz |  | QPSK | 12 |  |  |  |
| 15 MHz |  | QPSK | 16 |  |  |  |
| 20 MHz |  | QPSK | 18 |  |  |  |
| Note 1: V2X con-current band configurations are checked separately for each band configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2G.1-2. Note 2: The starting resource block for E-UTRA carrier and V2X carrier shall be RB #0+RB #0 and RB# (max+1 - RB allocation)+RB# (max+1 - RB allocation) of the channel bandwidth. Note 3: The Inter-band con-current V2X configurations and bandwidth combination sets in Table 5.4.2G.1-2, which UE supports, are tested across the uplink and v2x sidelink allocation combinations as above. Note 4: The test shall be performed with allocated sub-channel(s) at low and high end of the channel bandwidth respectively. |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A, in Figure A.93a.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3. Message content exceptions are defined in clause 6.5.2.2G.2.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL and V2X Reference Measurement channels are set according to Table 6.5.2.2G.2.4.1-1

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF-V2X according to TS 36.508 [7] clause 5.2A.2C. Message contents are defined in clause 6.5.2.2G.2.4.3.

6.5.2.2G.2.4.2	Test procedure

TP 1: V2X UE output power > 0dBm

1.	The SS configure the PSCCH and PSSCH transmission at the PCMAX,c,V2X level of the corresponding test point to enable V2X sidelink transmission power at 15dBm+/-10.5dB for carrier frequency  f > 5GHz.

2.	The SS simultaneously sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC and V2X scheduling information via PDCCH DCI format 5A for SL_V_RNTI to schedule the V2X RMC according to Table 6.5.2.2G.2.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL and V2X RMCs.

3.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE E-UTRA uplink output power is 13.2 dBm+/-3.2dB for carrier frequency f ≤ 3.0GHz or 13.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

4.	Measure the carrier leakage using Global In-Channel Tx-Test (Annex E) for E-UTRA and V2X respectively. On E-UTRA carrier, the TDD slots with transient periods are not under test.

TP 2: V2X UE output power within (-30, 0) dBm

1	The SS configure the PSCCH and PSSCH transmission at the PCMAX,c,V2X level of the corresponding test point to enable V2X sidelink transmission power at -15dBm+/-10.5dB for carrier frequency f > 5GHz.

2.	The SS simultaneously sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC and V2X scheduling information via PDCCH DCI format 5A for SL_V_RNTI to schedule the V2X RMC according to Table 6.5.2.2G.2.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL and V2X RMCs.

3.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE E-UTRA uplink output power is -36.8 dBm+/-3.2dB for carrier frequency f ≤ 3.0GHz or -36.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

4.	Measure the carrier leakage using Global In-Channel Tx-Test (Annex E) for E-UTRA and V2X respectively. On E-UTRA carrier, the TDD slots with transient periods are not under test.

6.5.2.2G.2.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the following exceptions:

Table 6.5.2.2G.2.4.3-1: SL-PSSCH-TxConfig-r14-DEFAULT

| Derivation Path: TS 36.508 [7] clause 4.6, Table 4.6.3-20F SL-PSSCH-TxConfig-r14-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| maxTxPower-r14 | 15 |  | TP1 |
|  | -15 |  | TP2 |

Table 6.5.2.2G.2.4.3-2: SL-CommResourcePoolV2X-r14-DEFAULT

| Derivation Path: TS 36.508 [7] clause 4.6, Table 4.6.3-20C: SL-CommResourcePoolV2X-r14-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| dataTxParameters-r14 SEQUENCE { |  |  |  |
| alpha-r12 | a0 |  |  |
| p0-r12 | 31 |  |  |
| } |  |  |  |

###### 6.5.2.2G.2.5	Test requirement

Each of the 20 carrier leakage results, derived in Annex E.4 for E-UTRA uplink and Annex E.9 for V2X sidelink respectively, shall not exceed the values in table 6.5.2.2G.2.5-1 and 6.5.2.2G.2.5-2.

Table 6.5.2.2G.2.5-1: Test requirements for Relative Carrier Leakage Power for E-UTRA

| LO Leakage | Parameters | Relative limit (dBc) | Applicable frequencies |
| --- | --- | --- | --- |
|  | f ≤ 3.0GHz: 13.2 dBm ±3.2dB 3.0GHz < f ≤ 4.2GHz: 13.5 dBm ±3.5dB | -27.2 | Carrier centre frequency < 1 GHz |
|  |  | -24.2 | Carrier centre frequency ≥ 1 GHz |
|  | f ≤ 3.0GHz: -36.8dBm±3.2dB 3.0GHz < f ≤ 4.2GHz: -36.5 dBm ±3.5dB | -9.2 |  |

Table 6.5.2.2G.2.5-2: Test requirements for Relative Carrier Leakage Power for V2X E-UTRA

| LO Leakage | Parameters | Relative Limit (dBc) |
| --- | --- | --- |
|  | Normal conditions:15 dBm ±10.5dB Extreme conditions: 15 dBm ±14.5dB | -24.2 |
|  | Normal conditions:-15 dBm ±10.5dB Extreme conditions: -15 dBm ±14.5dB | -19.2 |

##### 6.5.2.2G.3	Carrier leakage for V2X Communication / Intra-band contiguous MCC operation

###### 6.5.2.2G.3.1	Test Purpose

Same test purpose in clause 6.5.2.2.1.

###### 6.5.2.2G.3.2	Test applicability

This test case applies to all types of UE that supports Intra-band contiguous multi-carrier operation V2X Sidelink communication and Band 47.

###### 6.5.2.2G.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5.2.2G.0.

###### 6.5.2.2G.3.4	Test description

6.5.2.2G.3.4.1		Initial conditions

Initial conditions are a set of UE parameters that the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidth based on E-UTRA operating bands specified in table 5.4.2G.1-4. All these configurations shall be tested with applicable parameters for each channel bandwidth, and are shown in table 6.5.2.2G.3.4.1-1, the details of reference channel are specified in sub-clause A.8.3 and the reference GNSS configuration in TS 36.508 [7] subclause 4.11.

6.5.2.2G.3.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, High range |  |
| Test CC Combination setting(NRB_agg) as specified in subclause 5.4.2G |  |  | Lowest NRB_agg, Highest NRB_agg |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| V2X Multi-carrier Configuration |  | V2X PSSCH Allocation |  |  |
| CC1 NRB | CC2 NRB | CC1 & CC2 Mod |  | CC1 and CC2 RB Allocation |
| 50 | 50 | QPSK |  | 12@2+0@0 |
| 50 | 50 | QPSk |  | 0@0+12@37 |

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.89a.

2.	The parameter settings for the V2X sidelink transmission over PC5 are pre-configured according to TS 36.508 [7] subclause 4.10.1. Message content exceptions are defined in clause 6.5.2.2G.3.4.3.

3.	The V2X reference measurement channel is set according to Table 6.5.2.2G.3.4.1-1.

4.	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS36.508 [7] Table 4.11.2-2. Geographical area #1 is also pre-configured in the UEs.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 5A-V2X according to TS 36.508 [7] clause 4.5.9.

7.	The GNSS simulator is triggered to start step 1 of Scenario #1 to simulate a location in the centre of Geographical area #1. Wait for the UE to acquire the GNSS signal and start to transmit.

6.5.2.2G.3.4.2	Test procedure

TP 1: V2X UE output power > 0dBm

1	Set the V2X sidelink communication pre-configuration parameters for UE according to TS 36.508[7] Table 4.10.1.1-1 with the exception specified in table 6.5.2.2G.3.4.3-1 and Table 6.5.2.2G.3.4.3-2 to make sure V2X UE transmission power to be 15dBm+/-10.5dB for carrier frequency f > 5GHz;

2	The UE starts to perform the V2X sidelink communication according to SL-V2X-Preconfiguration. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the V2X RMC.

3.	Measure the carrier leakage using Global In-Channel Tx-Test (Annex E) in each component carrier respectively.

TP 2: V2X UE output power within (-30, 0) dBm

1	Set the V2X sidelink communication pre-configuration parameters for UE according to TS 36.508[7] Table 4.10.1.1-1 with the exception specified in table 6.5.2.2G.3.4.3-1 and Table 6.5.2.2G.3.4.3-2 to make sure V2X UE transmission power to be -15dBm+/-10.5dB for carrier frequency f > 5GHz;

2	The UE starts to perform the V2X sidelink communication according to SL-V2X-Preconfiguration. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the V2X RMC.

3.	Measure the carrier leakage using Global In-Channel Tx-Test (Annex E) in each component carrier respectively.

6.5.2.2G.3.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6, 4.7I and 4.10 with the following exceptions:

Table 6.5.2.2G.3.4.3-1: SL-V2X-PreconfigFreqInfo-r14-DEFAULT

| Derivation Path: 36.508 table 4.6.3-20K |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-V2X-PreconfigFreqInfo-r14-DEFAULT ::= SEQUENCE { |  |  |  |
| v2x-CommPreconfigGeneral-r14 SEQUENCE { |  | SL-PreconfigGeneral-r12 |  |
| maxTxPower-r12 | 15 |  | TP1 |
|  | -15 |  | TP2, |
| sl-bandwidth-r12 | n50 | 10Mhz |  |
| } |  |  |  |
| } |  |  |  |
| v2x-CommPreconfigSync-r14 SEQUENCE { |  | SL-PreconfigSync-r12 |  |
| syncOffsetIndicators-r14 SEQUENCE { |  |  |  |
| syncOffsetIndicator1-r14 | Not Present | Double synchronization signal transmission |  |
| } |  |  |  |
| } |  |  |  |
| v2x-CommRxPoolList-r14 SEQUENCE (SIZE (1..maxSL-V2X-RxPoolPreconf-r14)) OF SL-V2X-PreconfigCommPool-r14 { | 1 entry |  |  |
| SL-V2X-PreconfigCommPool-r14[1] | SL-V2X-PreconfigCommPool-r14-DEFAULT using condition BITMAP_6 and exception listed in Table Table 6.5.2.3G.1.4.3-2 |  |  |
| } |  |  |  |
| v2x-CommTxPoolList-r14 SEQUENCE (SIZE (1..maxSL-V2X-TxPoolPreconf-r14)) OF SL-V2X-PreconfigCommPool-r14 { |  |  |  |
| SL-V2X-PreconfigCommPool-r14[1] | SL-V2X-PreconfigCommPool-r14-DEFAULT using condition BITMAP_6 and exception listed in Table 6.5.2.3G.1.4.3-2 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.2.2G.3.4.3-2: SL-V2X-PreconfigCommPool-r14-DEFAULT

| Derivation Path: 36.508 clause Table 4.6.3-20J |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-CommResroucePoolV2X-r14-DEFAULT ::= SEQUENCE { |  |  |  |
| sl-Subframe-r14 CHOICE { |  |  |  |
| bs20-r14 | 11111111111111111111 |  | BITMAP_6 |
| } |  |  |  |
| sizeSubchannel-r14 | n15 |  | BW10 |
| numSubchannel-r14 | n1 |  |  |
| startRB-Subchannel-r14 | As required in the test configuration tables in subclause 6.5.2.3G.1.4.1 | The starting resource block shall be RB #0 and RB# (max+1 - RB allocation) of the channel bandwidth. |  |
| dataTxParameters-r14 SEQUENCE { |  |  |  |
| p0-r12 | 31 | It’s set to the maximum value to disable its effect on transmission power. |  |
| } |  |  |  |
| } |  |  |  |

###### 6.5.2.2G.3.5	Test requirement

Each of the 20 carrier leakage results at each component carrier, derived in Annex E.9, shall not exceed the values in table 6.5.2.2G.3.5-1.

Table 6.5.2.2G.3.5-1: Test requirements for Relative Carrier Leakage Power

| LO Leakage | Parameters | Relative Limit (dBc) |
| --- | --- | --- |
|  | Normal conditions:15 dBm ±10.5dB Extreme conditions: 15 dBm ±14.5dB | -24.2 |
|  | Normal conditions:-15 dBm ±10.5dB Extreme conditions: -15 dBm ±14.5dB | -19.2 |

#### 6.5.2.3	In-band emissions for non allocated RB

##### 6.5.2.3.1	Test Purpose

The in-band emissions are a measure of the interference falling into the non-allocated resources blocks

The in-band emission is defined as the average across 12 sub-carrier and as a function of the RB offset from the edge of the allocated UL transmission bandwidth. The in-band emission is measured as the ratio of the UE output power in a non–allocated RB to the UE output power in an allocated RB. The basic in-band emissions measurement interval is defined over one slot in the time domain. When the PUSCH or PUCCH transmission slot is shortened due to multiplexing with SRS, the in-band emissions measurement interval is reduced by one SC-FDMA symbol, accordingly. Likewise, when the PUSCH starting position is modified or when the second last symbol is the ending symbol of the PUSCH sub-frame for Frame Structure Type 3, the in-band emissions measurement interval is reduced accordingly.

##### 6.5.2.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 8 and forward.

##### 6.5.2.3.3	Minimum conformance requirements

The relative in-band emission shall not exceed the values specified in Tables 6.5.2.3.3-1 or 6.5.2.3.3-2.

Table 6.5.2.3.3-1: Minimum requirements for in-band emissions for UE supporting Rel-8 to Rel-10

| Parameter Description | Unit | Limit (Note 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | For Frame Structure Type 1 and Frame Structure Type 2: For Frame Structure Type 3 and 20 MHz channel bandwidth: max-10-6∆RB-1,-57 dBm/180 kHz-PRB |  | Any non-allocated (Note 2) |
| IQ Image | dB | -25 |  | Image frequencies (Notes 2, 3) |
| Carrier leakage | dBc | -25 | Output power > 0 dBm | LO frequency (Notes 4, 5) |
|  |  | -20 | -30 dBm ≤ Output power ≤ 0 dBm |  |
|  |  | -10 | -40 dBm Output power < -30 dBm |  |
| Note 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB - 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in Note 10. Note 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For Frame Structure Type 3 and 20 MHz channel bandwidth, the requirement applies for QPSK, 16QAM and 64QAM modulation with for any non-allocated RB andin the uplink scheduling grant with specified in [6]. Note 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the centre carrier frequency, but excluding any allocated RBs. For Frame Structure Type 3 with 20 MHz channel bandwidth, the applicable frequency is for QPSK, 16QAM and 64QAM modulation. Note 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. For Frame Structure Type 3 and 20 MHz channel bandwidth, the requirement applies for QPSK, 16QAM and 64QAM modulation within the uplink scheduling grant. Note 5: The applicable frequencies for this limit are those that are enclosed in the RBs containing the DC frequency if is odd, or in the two RBs immediately adjacent to the DC frequency if is even, but excluding any allocated RB. Note 6: is the Transmission Bandwidth (see Figure 5.4.2-1). Note 7: is the Transmission Bandwidth Configuration (see Figure 5.4.2-1). Note 8: is the limit specified in Table 6.5.2.1.1-1 for the modulation format used in the allocated RBs. Note 9: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. or for the first adjacent RB outside of the allocated bandwidth. Note 10: is the transmitted power per 180 kHz in allocated RBs, measured in dBm. |  |  |  |  |

Table 6.5.2.3.3-2: Minimum requirements for in-band emissions for UE supporting Rel.11 and higher

| Parameter description | Unit | Limit (Note 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB |  |  | Any non-allocated (Note 2) |
| IQ Image | dB | -28 | Image frequencies when carrier centre frequency < 1 GHz and Output power > 10 dBm | (Notes 2, 3) |
|  |  | -25 | Image frequencies when carrier centre frequency < 1 GHz and Output power ≤ 10 dBm |  |
|  |  | -25 | Image frequencies when carrier centre frequency ≥ 1 GHz |  |
| Carrier leakage | dBc | -28 | Output power > 10 dBm and carrier centre frequency < 1 GHz | Carrier frequency (Notes 4, 5) |
|  |  | -25 | Output power > 10 dBm and carrier centre frequency ≥ 1 GHz |  |
|  |  | -25 | 0 dBm ≤ Output power ≤10 dBm |  |
|  |  | -20 | -30 dBm ≤ Output power ≤ 0 dBm |  |
|  |  | -10 | -40 dBm Output power < -30 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB - 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in Note 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the centre carrier frequency, but excluding any allocated RBs. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit are those that are enclosed in the RBs containing the DC frequency if is odd, or in the two RBs immediately adjacent to the DC frequency if is even, but excluding any allocated RB. NOTE 6: is the Transmission Bandwidth (see Figure 5.6-1). NOTE 7: is the Transmission Bandwidth Configuration (see Figure 5.6-1). NOTE 8: is the limit specified in Table 6.5.2.1.1-1 for the modulation format used in the allocated RBs. NOTE 9: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. or for the first adjacent RB outside of the allocated bandwidth. NOTE 10: is the transmitted power per 180 kHz in allocated RBs, measured in dBm. |  |  |  |  |

The normative reference for this requirement is TS 36.101 [2] clause 6.5.2.3.1.

The in-band emission is defined as the average across 12 sub-carrier and as a function of the RB offset from the edge of the allocated UL transmission bandwidth. The in-band emission is measured as the ratio of the UE output power in a non–allocated RB to the UE output power in an allocated RB. The basic in-band emissions measurement interval is defined over one slot in the time domain.

##### 6.5.2.3.4	Test description

6.5.2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.5.2.3.4.1-1 and 6.5.2.3.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.2.3.4.1-1: Test Configuration Table for PUSCH

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  | See Table 6.5.1.4.1-1 |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  | See Table 6.5.1.4.1-1 |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  | See Table 6.5.1.4.1-1 |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
|  | Downlink Configuration | Uplink Configuration |  |  |
| Ch BW | N/A for in-band emissions testing | Mod'n | RB allocation |  |
|  |  |  | FDD | TDD |
| 1.4MHz |  | QPSK | 1 | 1 |
| 3MHz |  | QPSK | 4 | 4 |
| 5MHz |  | QPSK | 8 | 8 |
| 10MHz |  | QPSK | 12 | 12 |
| 15MHz |  | QPSK | 16 | 16 |
| 20MHz |  | QPSK | 18 | 18 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: For partial RB allocation, the starting resource block shall be RB #0 and RB# (max+1 - RB allocation) of the channel bandwidth. |  |  |  |  |

Table 6.5.2.3.4.1-2: Test Configuration Table for PUCCH

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  | See Table 6.5.1.4.1-1 |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 |  |  | See Table 6.5.1.4.1-1 |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | See Table 6.5.1.4.1-1 |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |
| Ch BW | Mod'n | RB allocation |  | FDD: PUCCH format = Format 1a TDD: PUCCH format = Format 1a / 1b |
|  |  | FDD | TDD |  |
| 1.4MHz | QPSK | 3 | 3 |  |
| 3MHz | QPSK | 4 | 4 |  |
| 5MHz | QPSK | 8 | 8 |  |
| 10MHz | QPSK | 16 | 16 |  |
| 15MHz | QPSK | 25 | 25 |  |
| 20MHz | QPSK | 30 | 30 |  |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Figure A.3.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL and DL Reference Measurement channels are set according to in Table 6.5.2.3.4.1-1 (PUSCH sub-test) and Table 6.5.2.3.4.1-2 (PUCCH sub-test).

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.5.2.3.4.3.

6.5.2.3.4.2	Test procedure

Test procedure for PUSCH:

1.1	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.5.2.3.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC

1.2	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is 13.2 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or 13.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

1.3	Measure In-band emission using Global In-Channel Tx-Test (Annex E)

1.4	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is 3.2 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or 3.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

1.5	Measure In-band emission using Global In-Channel Tx-Test (Annex E)

1.6	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is -26.8 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -26.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

1.7	Measure In-band emission using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test

1.8	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is to -36.8 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -36.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

1.9	Measure In-band emission using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test

Test procedure for PUCCH:

2.1	PUCCH is set according to Table 6.5.2.3.4.1-2. SS transmits PDSCH via PDCCH DCI format 1A for C_RNTI to transmit the DL RMC according to Table 6.5.2.3.4.1-2. The SS sends downlink MAC padding bits on the DL RMC. The transmission of PDSCH will make the UE send uplink ACK/NACK using PUCCH.

2.2	Send the appropriate TPC commands in the uplink scheduling information for PUCCH to the UE until UE output power is 13.2 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or 13.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

2.3	Measure In-band emission using Global In-Channel Tx-Test (Annex E)

2.4	Send the appropriate TPC commands in the uplink scheduling information for PUCCH to the UE until UE output power is 3.2 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or 3.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

2.5	Measure In-band emission using Global In-Channel Tx-Test (Annex E)

2.6	Send the appropriate TPC commands for PUCCH in the uplink scheduling information to the UE until UE output power is -26.8 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -26.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

2.7	Measure In-band emission using Global In-Channel Tx-Test (Annex E)

2.8	Send the appropriate TPC commands for PUCCH in the uplink scheduling information to the UE until UE output power is to -36.8 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -36.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

2.9	Measure In-band emission using Global In-Channel Tx-Test (Annex E)

NOTE:	The step 1.2, 1.3, 2.2 and 2.3 only apply to UE supporting Rel.11 and higher.

6.5.2.3.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the following exceptions:

Table 4.6.3-8: PUCCH-ConfigCommon: PUCCH in-band emissions measurement

| Derivation Path: 36.331 clause 6.3.2, Table 4.6.3-8: PUCCH-ConfigCommon-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-ConfigCommon-DEFAULT ::= SEQUENCE { |  |  |  |
| nRB-CQI | 0 |  |  |
| } |  |  |  |

##### 6.5.2.3.5	Test requirement

Each of the 20 In-band emissions results, derived in Annex E.4.3 shall not exceed the corresponding values in Tables 6.5.2.3.5-1 or 6.5.2.3.5-2 based on supported UE release version.

Table 6.5.2.3.5-1: Test requirements for in-band emissions for UE supporting Rel.8 to Rel.10

| Parameter Description | Unit | Limit (Note 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | +0.8 |  | Any non-allocated (Note 2) |
| IQ Image | dB | -24.2 |  | Image frequencies (Notes 2, 3) |
| Carrier leakage | dBc | -24.2 | Output power f ≤ 3.0GHz: 3.2dBm ±3.2dB 3.0GHz < f ≤ 4.2GHz: 3.5 dBm ±3.5dB | LO frequency (Notes 4, 5) |
|  |  | -19.2 | Output power f ≤ 3.0GHz: -26.8 dBm ±3.2dB 3.0GHz < f ≤ 4.2GHz: -26.5 dBm ±3.5dB |  |
|  |  | -9.2 | Output power f ≤ 3.0GHz: -36.8 dBm ±3.2dB 3.0GHz < f ≤ 4.2GHz: -36.5 dBm ±3.5dB |  |
| Note 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the test requirement is calculated as the higher of PRB – 29.2 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in Note 10. Note 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. Note 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the centre carrier frequency, but excluding any allocated RBs. Note 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. Note 5: The applicable frequencies for this limit are those that are enclosed in the RBs containing the DC frequency if is odd, or in the two RBs immediately adjacent to the DC frequency if is even, but excluding any allocated RB. Note 6: is the Transmission Bandwidth (see Figure 5.4.2-1). Note 7: is the Transmission Bandwidth Configuration (see Figure 5.4.2-1). Note 8: is the limit specified in Table 6.5.2.1.1-1 for the modulation format used in the allocated RBs. Note 9: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. or for the first adjacent RB outside of the allocated bandwidth. Note 10: is the transmitted power per 180 kHz in allocated RBs, measured in dBm. |  |  |  |  |

Table 6.5.2.3.5-2: Test requirements for in-band emissions for UE supporting Rel.11 and higher

| Parameter description | Unit | Limit (Note 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | +0.8 |  | Any non-allocated (Note 2) |
| IQ Image | dB | -27.2 | Image frequencies when carrier centre frequency < 1 GHz and Output power > 10 dBm | (Notes 2, 3) |
|  |  | -24.2 | Image frequencies when carrier centre frequency < 1 GHz and Output power ≤ 10 dBm |  |
|  |  | -24.2 | Image frequencies when carrier centre frequency ≥ 1 GHz |  |
| Carrier leakage | dBc | -27.2 | Output power > 10 dBm and carrier centre frequency < 1 GHz | Carrier frequency (Notes 4, 5) |
|  |  | -24.2 | Output power > 10 dBm and carrier centre frequency ≥ 1 GHz |  |
|  |  | -24.2 | 0 dBm ≤ Output power ≤10 dBm |  |
|  |  | -19.2 | -30 dBm ≤ Output power ≤ 0 dBm |  |
|  |  | -9.2 | -40 dBm Output power < -30 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB - 29.2 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in Note 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the centre carrier frequency, but excluding any allocated RBs. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit are those that are enclosed in the RBs containing the DC frequency if is odd, or in the two RBs immediately adjacent to the DC frequency if is even, but excluding any allocated RB. NOTE 6: is the Transmission Bandwidth (see Figure 5.4.2-1). NOTE 7: is the Transmission Bandwidth Configuration (see Figure 5.4.2-1). NOTE 8: is the limit specified in Table 6.5.2.1.1-1 for the modulation format used in the allocated RBs. NOTE 9: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. or for the first adjacent RB outside of the allocated bandwidth. NOTE 10: is the transmitted power per 180 kHz in allocated RBs, measured in dBm. |  |  |  |  |

#### 6.5.2.3E	In-band emissions for non allocated RB for UE category 0

##### 6.5.2.3E.1	Test Purpose

Same test purpose as in clause 6.5.2.3.1.

##### 6.5.2.3E.2	Test applicability

This test case applies to all types of E-UTRA UE release 12 and forward of UE category 0.

##### 6.5.2.3E.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.2.3.3.

##### 6.5.2.3E.4	Test description

6.5.2.3E.4.1	Initial conditions

Same initial conditions as in clause 6.5.2.3.4.1 with following exceptions:

-	Connect SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.3 using only main UE Tx/Rx antenna.

6.5.2.3E.4.2	Test procedure

Same test procedure as in clause 6.5.2.3.4.2

6.5.2.3E.4.3	Message contents

Same message contents as in clause 6.5.2.3.4.3.

##### 6.5.2.3E.5	Test requirement

Each of the 20 In-band emissions results, derived in Annex E.4.3 shall not exceed the corresponding values in Table 6.5.2.3E.5-1

Table 6.5.2.3E.5-1: Test requirements for in-band emissions

| Parameter Description | Unit | Limit (Note 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | +0.8 |  | Any non-allocated (Note 2) |
| IQ Image | dB | -24.2 |  | Image frequencies (Notes 2, 3) |
| DC | dBc | -24.2 | Output power f ≤ 3.0GHz: 3.2dBm ±3.2dB 3.0GHz < f ≤ 4.2GHz: 3.5 dBm ±3.5dB | LO frequency (Notes 4, 5) |
|  |  | -19.2 | Output power f ≤ 3.0GHz: -26.8 dBm ±3.2dB 3.0GHz < f ≤ 4.2GHz: -26.5 dBm ±3.5dB |  |
|  |  | -9.2 | Output power f ≤ 3.0GHz: -36.8 dBm ±3.2dB 3.0GHz < f ≤ 4.2GHz: -36.5 dBm ±3.5dB |  |
| Note 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the test requirement is calculated as the higher of PRB – 29.2 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in Note 10. Note 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. Note 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the centre carrier frequency, but excluding any allocated RBs. Note 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. Note 5: The applicable frequencies for this limit are those that are enclosed in the RBs containing the DC frequency if is odd, or in the two RBs immediately adjacent to the DC frequency if is even, but excluding any allocated RB. Note 6: is the Transmission Bandwidth (see Figure 5.4.2-1). Note 7: is the Transmission Bandwidth Configuration (see Figure 5.4.2-1). Note 8: is the limit specified in Table 6.5.2.1.1-1 for the modulation format used in the allocated RBs. Note 9: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. or for the first adjacent RB outside of the allocated bandwidth. Note 10: is the transmitted power per 180 kHz in allocated RBs, measured in dBm. |  |  |  |  |

#### 6.5.2.3EA	In-band emissions for non allocated RB for UE category M1

##### 6.5.2.3EA.1	Test Purpose

Same test purpose as in clause 6.5.2.3.1.

##### 6.5.2.3EA.2	Test applicability

This test case applies to all types of E-UTRA UE release 13 and forward of UE category M1.

##### 6.5.2.3EA.3	Minimum conformance requirements

The relative in-band emission when centre carrier frequency is at the centre of channel bandwidth or when at the 6RB narrowband assigned for transmission shall not exceed the values specified in Table 6.5.2.3EA.3-1 or Table 6.5.2.3EA.3-2.

Table 6.5.2.3EA.3-1: Minimum requirements for in-band emissions for UE supporting Rel-13 and Rel-14

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB |  |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -28 | Image frequencies when carrier centre frequency < 1 GHz and Output power > 10 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -25 | Image frequencies when carrier centre frequency < 1 GHz and Output power ≤ 10 dBm |  |
|  |  | -25 | Image frequencies when carrier centre frequency ≥ 1 GHz |  |
| Carrier leakage | dBc | -28 | Output power > 10 dBm and carrier centre frequency < 1 GHz | Carrier frequency (NOTES 4, 5) |
|  |  | -25 | Output power > 10 dBm and carrier centre frequency ≥ 1 GHz |  |
|  |  | -25 | 0 dBm ≤ Output power ≤10 dBm |  |
|  |  | -20 | -30 dBm ≤ Output power ≤ 0 dBm |  |
|  |  | -10 | -40 dBm Output power < -30 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB - 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the centre carrier frequency, but excluding any allocated RBs. For UE of UL Category M1, applicable frequencies shall alternatively include those found by reflection on the centre of the assigned 6 RB narrowband, but excluding any allocated RBs. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit are those that are enclosed in the RBs containing the DC frequency if is odd, or in the two RBs immediately adjacent to the DC frequency if is even, but excluding any allocated RB. For UE of UL Category M1, the applicable frequencies shall alternatively be the centre frequency of the supported 6RBs additionally. NOTE 6: is the Transmission Bandwidth (see Figure 5.6-1). NOTE 7: is the Transmission Bandwidth Configuration (see Figure 5.6-1). NOTE 8: is the limit specified in Table 6.5.2.1.1-1 for the modulation format used in the allocated RBs. NOTE 9: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. or for the first adjacent RB outside of the allocated bandwidth. NOTE 10: is the transmitted power per 180 kHz in allocated RBs, measured in dBm. |  |  |  |  |

Table 6.5.2.3EA.3-2: Minimum requirements for in-band emissions for UE supporting Rel-15 and higher

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | -18-5∙¦∆SubG-1)/LSCG |  | Any non-allocated Subcarrier Group within the subPRB allocation (NOTE 11,12,13) |
|  |  |  |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -28 | Image frequencies when carrier centre frequency < 1 GHz and Output power > 10 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -25 | Image frequencies when carrier centre frequency < 1 GHz and Output power ≤ 10 dBm |  |
|  |  | -25 | Image frequencies when carrier centre frequency ≥ 1 GHz |  |
| Carrier leakage | dBc | -28 | Output power > 10 dBm and carrier centre frequency < 1 GHz | Carrier frequency (NOTES 4, 5) |
|  |  | -25 | Output power > 10 dBm and carrier centre frequency ≥ 1 GHz |  |
|  |  | -25 | 0 dBm ≤ Output power ≤10 dBm |  |
|  |  | -20 | -30 dBm ≤ Output power ≤ 0 dBm |  |
|  |  | -10 | -40 dBm Output power < -30 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB - 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the centre carrier frequency, but excluding any allocated RBs. For UE of UL Category M1, applicable frequencies shall alternatively include those found by reflection on the centre of the assigned 6 RB narrowband, but excluding any allocated RBs. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit are those that are enclosed in the RBs containing the DC frequency if is odd, or in the two RBs immediately adjacent to the DC frequency if is even, but excluding any allocated RB. For UE of UL Category M1, the applicable frequencies shall alternatively be the centre frequency of the supported 6RBs additionally. NOTE 6: is the Transmission Bandwidth (see Figure 5.6-1). NOTE 7: is the Transmission Bandwidth Configuration (see Figure 5.6-1). NOTE 8: is the limit specified in Table 6.5.2.1.1-1 for the modulation format used in the allocated RBs. NOTE 9: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. or for the first adjacent RB outside of the allocated bandwidth. NOTE 10: is the transmitted power per 180 kHz in allocated RBs, measured in dBm. NOTE 11: The measurement bandwidth is 1 subcarrier group ( [3] subcarrier per subcarrier group) and the limit is expressed as a ratio of measured power in one non-allocated subcarrier group to the measured total power in all allocated subcarrier. NOTE 12: ∆SubG is the starting frequency offset between the allocated subcarrier group and the measured non-allocated subcarrier group (e.g. ∆SubG=1 or ∆SubG =-1 for the first adjacent subcarrier group outside the allocated subcarrier group.) NOTE 13: LSCG is the Transmission bandwidth (number of subcarrier group). |  |  |  |  |

The in-band emission is defined as the average across 12 sub-carrier and as a function of the RB offset from the edge of the allocated UL transmission bandwidth. The in-band emission is measured as the ratio of the UE output power in a non–allocated RB to the UE output power in an allocated RB. The basic in-band emissions measurement interval is defined over one slot in the time domain.

The normative reference for this requirement is TS 36.101 [2] clause 6.5.2E.3.1.

##### 6.5.2.3EA.4	Test description

6.5.2.3EA.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in sub-clause 5.2E. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.5.2.3EA.4.1-1 and 6.5.2.3EA.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and MPDCCH before measurement are specified in Annex C.2.

Table 6.5.2.3EA.4.1-1: Test Configuration Table for PUSCH

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  | See Table 6.5.1.4.1-1 |  |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  | See Table 6.5.1.4.1-1 |  |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  | 5MHz |  |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  | Downlink Configuration | Uplink Configuration |  |  |  |
| Ch BW | N/A for in-band emissions testing | Mod'n | RB allocation |  |  |
|  |  |  | FDD and HD-FDD | TDD | Narrowband index1 |
| 5MHz |  | QPSK | 1 | 1 | 0 |
| 5MHz |  | QPSK | ½ (Note 3) | ½ (Note 3) | 0 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4. Note 2: The RBstart of partial RB allocation shall be RB#0 and RB# (6 - RB allocation) of the narrowband. Note 3: Applies only for UE supporting sub-PRB resource allocation for PUSCH. |  |  |  |  |  |

Table 6.5.2.3EA.4.1-2: Test Configuration Table for PUCCH

|  |  |  | Initial Conditions |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  | See Table 6.5.1.4.1-1 |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 |  |  | See Table 6.5.1.4.1-1 |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 5MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  | Downlink Configuration |  |  |  | Uplink Configuration |
| Ch BW | Mod'n | RB allocation |  |  | FDD: PUCCH format = Format 1a TDD: PUCCH format = Format 1a / 1b |
|  |  | FDD | TDD | Narrowband index1 |  |
| 5MHz | QPSK | 4@0 | 4@0 | 0 |  |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4. |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL and DL Reference Measurement channels are set according to in Table 6.5.2.3.4.1-1 (PUSCH sub-test) and Table 6.5.2.3EA.4.1-2 (PUCCH sub-test).

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [7] clause 5.2A.2AA. Message contents are defined in clause 6.5.2.3EA.4.3.

6.5.2.3EA.4.2	Test procedure

Test procedure for PUSCH:

1.1	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.5.2.3EA.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC

1.2	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is 13.2 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or 13.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

1.3	Measure In-band emission using Global In-Channel Tx-Test (Annex E) according to the UE’s declaration on the position of carrier centre frequency.

1.4	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is 3.2 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or 3.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

1.5	Measure In-band emission using Global In-Channel Tx-Test (Annex E) according to the UE’s declaration on the position of carrier centre frequency.

1.6	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is -26.8 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -36.5dBm ±2.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

1.7	Measure In-band emission using Global In-Channel Tx-Test (Annex E) according to the UE’s declaration on the position of carrier centre frequency. For TDD slots with transient periods are not under test. For HD-FDD slots with transient periods are not under test. Half-duplex guard subframes are not under test.

1.8	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is to -36.8 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -36.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

1.9	Measure In-band emission using Global In-Channel Tx-Test (Annex E) according to the UE’s declaration on the position of carrier centre frequency. For TDD slots with transient periods are not under test. For HD-FDD slots with transient periods are not under test. Half-duplex guard subframes are not under test.

Test procedure for PUCCH:

2.1	PUCCH is set according to Table 6.5.2.3EA.4.1-2. SS transmits PDSCH via MPDCCH DCI format 6-1A for C_RNTI to transmit the DL RMC according to Table 6.5.2.3EA.4.1-2. The SS sends downlink MAC padding bits on the DL RMC. The transmission of PDSCH will make the UE send uplink ACK/NACK using PUCCH.

2.2	Send the appropriate TPC commands in the uplink scheduling information for PUCCH to the UE until UE output power is 13.2 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or 13.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

2.3	Measure In-band emission using Global In-Channel Tx-Test (Annex E) according to the UE’s declaration on the position of carrier centre frequency.

2.4	Send the appropriate TPC commands in the uplink scheduling information for PUCCH to the UE until UE output power is 3.2 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or 3.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

2.5	Measure In-band emission using Global In-Channel Tx-Test (Annex E) according to the UE’s declaration on the position of carrier centre frequency.

2.6	Send the appropriate TPC commands for PUCCH in the uplink scheduling information to the UE until UE output power is -26.8 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -26.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

2.7	Measure In-band emission using Global In-Channel Tx-Test (Annex E) according to the UE’s declaration on the position of carrier centre frequency.

2.8	Send the appropriate TPC commands for PUCCH in the uplink scheduling information to the UE until UE output power is to -36.8 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -36.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

2.9	Measure In-band emission using Global In-Channel Tx-Test (Annex E) according to the UE’s declaration on the position of carrier centre frequency.

6.5.2.3EA.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the condition CEModeA and the following exceptions:

Table 6.5.2.3EA.4.3-1: PUCCH-ConfigCommon: PUCCH in-band emissions measurement

| Derivation Path: TS 36.508 [7] clause 6.3.2, Table 4.6.3-8: PUCCH-ConfigCommon-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-ConfigCommon-DEFAULT ::= SEQUENCE { |  |  |  |
| nRB-CQI | 0 |  |  |
| } |  |  |  |

##### 6.5.2.3EA.5	Test requirement

Each of the 20 In-band emissions results, derived in Annex E.4.3 shall not exceed the corresponding values in Table 6.5.2.3EA.5-1 or Table 6.5.2.3EA.5-2.

Table 6.5.2.3EA.5-1: Test requirements for in-band emissions for UE supporting Rel-13 and Rel-14

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | +0.8 |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -27.2 | Image frequencies when carrier centre frequency < 1 GHz and Output power > 10 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -24.2 | Image frequencies when carrier centre frequency < 1 GHz and Output power ≤ 10 dBm |  |
|  |  | -24.2 | Image frequencies when carrier centre frequency ≥ 1 GHz |  |
| Carrier leakage | dBc | -27.2 | Output power > 10 dBm and carrier centre frequency < 1 GHz | Carrier frequency (NOTES 4, 5) |
|  |  | -24.2 | Output power > 10 dBm and carrier centre frequency ≥ 1 GHz |  |
|  |  | -24.2 | 0 dBm ≤ Output power ≤10 dBm |  |
|  |  | -19.2 | -30 dBm ≤ Output power ≤ 0 dBm |  |
|  |  | -9.2 | -40 dBm Output power < -30 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB - 29.2 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the centre carrier frequency, but excluding any allocated RBs. For UE of UL Category M1, applicable frequencies shall alternatively include those found by reflection on the centre of the assigned 6 RB narrowband, but excluding any allocated RBs. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit are those that are enclosed in the RBs containing the DC frequency if is odd, or in the two RBs immediately adjacent to the DC frequency if is even, but excluding any allocated RB. For UE of UL Category M1, the applicable frequencies shall alternatively be the centre frequency of the supported 6RBs additionally. NOTE 6: is the Transmission Bandwidth (see Figure 5.6-1). NOTE 7: is the Transmission Bandwidth Configuration (see Figure 5.6-1). NOTE 8: is the limit specified in Table 6.5.2.1.1-1 for the modulation format used in the allocated RBs. NOTE 9: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. or for the first adjacent RB outside of the allocated bandwidth. NOTE 10: is the transmitted power per 180 kHz in allocated RBs, measured in dBm. |  |  |  |  |

Table 6.5.2.3EA.5-2: Test requirements for in-band emissions for UE supporting Rel-15 and higher

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | -18-5∙¦∆SubG-1)/LSCG |  | Any non-allocated Subcarrier Group within the subPRB allocation (NOTE 11,12,13) |
|  |  |  |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -28 | Image frequencies when carrier centre frequency < 1 GHz and Output power > 10 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -25 | Image frequencies when carrier centre frequency < 1 GHz and Output power ≤ 10 dBm |  |
|  |  | -25 | Image frequencies when carrier centre frequency ≥ 1 GHz |  |
| Carrier leakage | dBc | -28 | Output power > 10 dBm and carrier centre frequency < 1 GHz | Carrier frequency (NOTES 4, 5) |
|  |  | -25 | Output power > 10 dBm and carrier centre frequency ≥ 1 GHz |  |
|  |  | -25 | 0 dBm ≤ Output power ≤10 dBm |  |
|  |  | -20 | -30 dBm ≤ Output power ≤ 0 dBm |  |
|  |  | -10 | -40 dBm Output power < -30 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB - 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the centre carrier frequency, but excluding any allocated RBs. For UE of UL Category M1, applicable frequencies shall alternatively include those found by reflection on the centre of the assigned 6 RB narrowband, but excluding any allocated RBs. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit are those that are enclosed in the RBs containing the DC frequency if is odd, or in the two RBs immediately adjacent to the DC frequency if is even, but excluding any allocated RB. For UE of UL Category M1, the applicable frequencies shall alternatively be the centre frequency of the supported 6RBs additionally. NOTE 6: is the Transmission Bandwidth (see Figure 5.6-1). NOTE 7: is the Transmission Bandwidth Configuration (see Figure 5.6-1). NOTE 8: is the limit specified in Table 6.5.2.1.1-1 for the modulation format used in the allocated RBs. NOTE 9: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. or for the first adjacent RB outside of the allocated bandwidth. NOTE 10: is the transmitted power per 180 kHz in allocated RBs, measured in dBm. NOTE 11: The measurement bandwidth is 1 subcarrier group ( [3] subcarrier per subcarrier group) and the limit is expressed as a ratio of measured power in one non-allocated subcarrier group to the measured total power in all allocated subcarrier. NOTE 12: ∆SubG is the starting frequency offset between the allocated subcarrier group and the measured non-allocated subcarrier group (e.g. ∆SubG=1 or ∆SubG =-1 for the first adjacent subcarrier group outside the allocated subcarrier group.) NOTE 13: LSCG is the Transmission bandwidth (number of subcarrier group). |  |  |  |  |

#### 6.5.2.3EB	In-band emissions for non allocated RB for UE category 1bis

##### 6.5.2.3EB.1	Test purpose

Same test purpose as in clause 6.5.2.3.

##### 6.5.2.3EB.2	Test applicability

This test applies to all types of E-UTRA UE release 13 and forward of UE category 1bis.

##### 6.5.2.3EB.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.2.3.3.

##### 6.5.2.3EB.4	Test description

6.5.2.3EB.4.1	Initial conditions

Same initial conditions as in clause 6.5.2.3.4.1 with the following exception:

-	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.3 using only main UE Tx/Rx antenna.

6.5.2.3EB.4.2	Test procedure

Same test procedure as in clause 6.5.2.3.4.2.

6.5.2.3EB.4.3	Message contents

Same message contents as in clause 6.5.2.3.4.3.

##### 6.5.2.3EB.5	Test requirement

Each of the 20 In-band emissions results, derived in Annex E.4.3 shall not exceed the corresponding values in Tables 6.5.2.3EB.5-1 based on supported UE release version.

Table 6.5.2.3EB.5-1: Test requirements for in-band emissions for UE supporting Rel.13 and higher

| Parameter description | Unit | Limit (Note 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | +0.8 |  | Any non-allocated (Note 2) |
| IQ Image | dB | -27.2 | Image frequencies when carrier centre frequency < 1 GHz and Output power > 10 dBm | (Notes 2, 3) |
|  |  | -24.2 | Image frequencies when carrier centre frequency < 1 GHz and Output power ≤ 10 dBm |  |
|  |  | -24.2 | Image frequencies when carrier centre frequency ≥ 1 GHz |  |
| Carrier leakage | dBc | -27.2 | Output power > 10 dBm and carrier centre frequency < 1 GHz | Carrier frequency (Notes 4, 5) |
|  |  | -24.2 | Output power > 10 dBm and carrier centre frequency ≥ 1 GHz |  |
|  |  | -24.2 | 0 dBm ≤ Output power ≤10 dBm |  |
|  |  | -19.2 | -30 dBm ≤ Output power ≤ 0 dBm |  |
|  |  | -9.2 | -40 dBm Output power < -30 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB - 29.2 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in Note 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the centre carrier frequency, but excluding any allocated RBs. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit are those that are enclosed in the RBs containing the DC frequency if is odd, or in the two RBs immediately adjacent to the DC frequency if is even, but excluding any allocated RB. NOTE 6: is the Transmission Bandwidth (see Figure 5.4.2-1). NOTE 7: is the Transmission Bandwidth Configuration (see Figure 5.4.2-1). NOTE 8: is the limit specified in Table 6.5.2.1.1-1 for the modulation format used in the allocated RBs. NOTE 9: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. or for the first adjacent RB outside of the allocated bandwidth. NOTE 10: is the transmitted power per 180 kHz in allocated RBs, measured in dBm. |  |  |  |  |

#### 6.5.2.3EC	In-band emissions for non allocated RB for UE category M2

##### 6.5.2.3EC.1	Test Purpose

Same test purpose as in clause 6.5.2.3.1.

##### 6.5.2.3EC.2	Test applicability

This test case applies to all types of E-UTRA UE release 14 and forward of UE category M2.

##### 6.5.2.3EC.3	Minimum conformance requirements

The relative in-band emission when the center carrier frequency is either at the center of channel bandwidth or at the center of the narrowband assigned for transmission shall not exceed the values specified in Table 6.5.2.3EC.3-1

Table 6.5.2.3EC.3-1: Minimum requirements for in-band emissions

| Parameter description |  | Unit |  | Limit (NOTE 1) |  |  | Applicable Frequencies |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| General |  | dB |  |  |  |  | Any non-allocated Subcarrier Group within the subPRB allocation (NOTE 11,12,13) |  |
|  |  |  |  |  |  |  | Any non-allocated (NOTE 2) |  |
| IQ Image |  | dB |  | -28 |  | Image frequencies when carrier center frequency < 1 GHz and Output power > 10 dBm | Image frequencies (NOTES 2, 3) |  |
|  |  |  |  | -25 |  | Image frequencies when carrier center frequency < 1 GHz and Output power ≤ 10 dBm |  |  |
|  |  |  |  | -25 |  | Image frequencies when carrier center frequency ≥ 1 GHz |  |  |
| Carrier leakage |  | dBc |  | -28 |  | Output power > 10 dBm and carrier center frequency < 1 GHz | Carrier frequency (NOTES 4, 5) |  |
|  |  |  |  | -25 |  | Output power > 10 dBm and carrier center frequency ≥ 1 GHz |  |  |
|  |  |  |  | -25 |  | 0 dBm ≤ Output power ≤10 dBm |  |  |
|  |  |  |  | -20 |  | -30 dBm ≤ Output power ≤ 0 dBm |  |  |
|  |  |  |  | -10 |  | -40 dBm Output power < -30 dBm |  |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB - 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the centre carrier frequency, but excluding any allocated RBs. For UE of UL Categories M1 and M2, the applicable frequencies shall alternatively include those found by reflection on the center of the assigned narrowband, but excluding any allocated RBs. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit are those that are enclosed in the RBs containing the DC frequency if is odd, or in the two RBs immediately adjacent to the DC frequency if is even, but excluding any allocated RB. For UE of UL Categories M1 and M2, the applicable frequencies shall alternatively be the centre frequency of the supported 6RBs. NOTE 6: is the Transmission Bandwidth (see Figure 5.4.2-1). NOTE 7: is the Transmission Bandwidth Configuration (see Figure 5.4.2-1). NOTE 8: is the limit specified in Table 6.5.2.1.3-1 for the modulation format used in the allocated RBs. NOTE 9: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. or for the first adjacent RB outside of the allocated bandwidth. NOTE 10: is the transmitted power per 180 kHz in allocated RBs, measured in dBm. NOTE 11: The measurement bandwidth is 1 subcarrier group ( [3] subcarrier per subcarrier group) and the limit is expressed as a ratio of measured power in one non-allocated subcarrier group to the measured total power in all allocated subcarrier. NOTE 12: ∆SubG is the starting frequency offset between the allocated subcarrier group and the measured non-allocated subcarrier group (e.g. ∆SubG=1 or ∆SubG =-1 for the first adjacent subcarrier group outside the allocated subcarrier group.) NOTE 13: LSCG is the Transmission bandwidth (number of subcarrier group). |  |  |  |  |  |  |  |  |

The in-band emission is defined as the average across 12 sub-carrier and as a function of the RB offset from the edge of the allocated UL transmission bandwidth. The in-band emission is measured as the ratio of the UE output power in a non–allocated RB to the UE output power in an allocated RB.

The basic in-band emissions measurement interval is defined over one slot in the time domain.

The normative reference for this requirement is TS 36.101 [2] clause 6.5.2E.3.1.

##### 6.5.2.3EC.4	Test description

6.5.2.3EC.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in sub-clause 5.2E. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.5.2.3EC.4.1-1 and 6.5.2.3EC.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and MPDCCH before measurement are specified in Annex C.2.

Table 6.5.2.3EC.4.1-1: Test Configuration Table for PUSCH

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  | Low range, Mid range, High range |  |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  | 5MHz, Highest |  |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  | Downlink Configuration | Uplink Configuration |  |  |  |
| Ch BW | N/A for in-band emissions testing | Mod'n | RB allocation |  |  |
|  |  |  | FDD and HD-FDD | TDD | Narrowband index1 |
| Low range, Mid range |  |  |  |  |  |
| 5MHz |  | QPSK | 1 | 1 | 0 |
| 5MHz |  | QPSK | 24 | 24 | 0 |
| 10MHz |  | QPSK | 1 | 1 | 0 |
| 10MHz |  | QPSK | 24 | 24 | 0 |
| 15MHz |  | QPSK | 1 | 1 | 0 |
| 15MHz |  | QPSK | 24 | 24 | 0 |
| 20MHz |  | QPSK | 1 | 1 | 0 |
| 20MHz |  | QPSK | 24 | 24 | 0 |
| High range |  |  |  |  |  |
| 5MHz |  | QPSK | 1 | 1 | 3 |
| 5MHz |  | QPSK | 24 | 24 | 0 |
| 10MHz |  | QPSK | 1 | 1 | 7 |
| 10MHz |  | QPSK | 24 | 24 | 4 |
| 15MHz |  | QPSK | 1 | 1 | 11 |
| 15MHz |  | QPSK | 24 | 24 | 8 |
| 20MHz |  | QPSK | 1 | 1 | 15 |
| 20MHz |  | QPSK | 24 | 24 | 12 |
| Note 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.7. Note 2: The RBstart of the 1RB allocation shall be RB#0 of the narrowband for Low/Mid frequency range and RB#5 of the narrowband for High frequency range. |  |  |  |  |  |

Table 6.5.2.3EC.4.1-2: Test Configuration Table for PUCCH

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 5MHz, Highest |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  | Downlink Configuration |  |  |  | Uplink Configuration |
| Ch BW | Mod'n | RB allocation |  |  |  |
|  |  | FDD | TDD | Narrowband index1 |  |
| Low range, Mid range |  |  |  |  |  |
| 5MHz | QPSK | 12@0 | 12@0 | 0 | FDD: PUCCH format = Format 1a TDD: PUCCH format = Format 1a / 1b |
| 10MHz | QPSK | 12@0 | 12@0 | 0 |  |
| 15MHz | QPSK | 12@0 | 12@0 | 0 |  |
| 20MHz | QPSK | 12@0 | 12@0 | 0 |  |
| High range |  |  |  |  |  |
| 5MHz | QPSK | 12@0 | 12@0 | 0 | FDD: PUCCH format = Format 1a TDD: PUCCH format = Format 1a / 1b |
| 10MHz | QPSK | 12@0 | 12@0 | 4 |  |
| 15MHz | QPSK | 12@0 | 12@0 | 8 |  |
| 20MHz | QPSK | 12@0 | 12@0 | 12 |  |
| Note 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.7. |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL and DL Reference Measurement channels are set according to in Table 6.5.2.3EC.4.1-1 (PUSCH sub-test) and Table 6.5.2.3EC.4.1-2 (PUCCH sub-test).

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [7] clause 5.2A.2AA. Message contents are defined in clause 6.5.2.3EC.4.3.

6.5.2.3EC.4.2	Test procedure

Same test procedure as in clause 6.5.2.2EA.4.2 with the following exception.

-	Instead of Table 6.5.2.3EA.4.1-1  use Table 6.5.2.3EC.4.1-1.

-	Instead of Table 6.5.2.3EA.4.1-2  use Table 6.5.2.3EC.4.1-2.

6.5.2.3EC.4.3	Message contents

Same message contents as in clause 6.5.2.3EA.4.3.

##### 6.5.2.3EC.5	Test requirement

Each of the 20 In-band emissions results, derived in Annex E.4.3 shall not exceed the corresponding values in Table 6.5.2.3EC.5-1

Table 6.5.2.3EC.5-1: Test requirements for in-band emissions

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB |  |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -27.2 | Image frequencies when carrier center frequency < 1 GHz and Output power > 10 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -24.2 | Image frequencies when carrier center frequency < 1 GHz and Output power ≤ 10 dBm |  |
|  |  | -24.2 | Image frequencies when carrier center frequency ≥ 1 GHz |  |
| Carrier leakage | dBc | -27.2 | Output power > 10 dBm and carrier center frequency < 1 GHz | Carrier frequency (NOTES 4, 5) |
|  |  | -24.2 | Output power > 10 dBm and carrier center frequency ≥ 1 GHz |  |
|  |  | -24.2 | 0 dBm ≤ Output power ≤10 dBm |  |
|  |  | -19.2 | -30 dBm ≤ Output power ≤ 0 dBm |  |
|  |  | -9.2 | -40 dBm Output power < -30 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB - 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the centre carrier frequency, but excluding any allocated RBs. For UE of UL Categories M1 and M2, the applicable frequencies shall alternatively include those found by reflection on the center of the assigned narrowband, but excluding any allocated RBs. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit are those that are enclosed in the RBs containing the DC frequency if is odd, or in the two RBs immediately adjacent to the DC frequency if is even, but excluding any allocated RB. For UE of UL Categories M1 and M2, the applicable frequencies shall alternatively be the centre frequency of the supported 6RBs. NOTE 6: is the Transmission Bandwidth (see Figure 5.4.2-1). NOTE 7: is the Transmission Bandwidth Configuration (see Figure 5.4.2-1). NOTE 8: is the limit specified in Table 6.5.2.1.3-1 for the modulation format used in the allocated RBs. NOTE 9: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. or for the first adjacent RB outside of the allocated bandwidth. NOTE 10: is the transmitted power per 180 kHz in allocated RBs, measured in dBm. |  |  |  |  |

#### 6.5.2.3F	In-band emissions for non allocated RB for category NB1 and NB2

6.5.2.3F.1	Test purpose

The in-band emissions are a measure of the interference falling into the non-allocated tones.

The in-band emission is defined as a function of the tone offset from the edge of the allocated UL transmission tone(s) within the transmission bandwidth configuration. The in-band emission is measured as the ratio of the UE output power in a non–allocated tone to the UE output power in an allocated tone. The basic in-band emissions measurement interval is defined over one slot in the time domain.

6.5.2.3F.2	Test applicability

This test case applies to all types of E-UTRA FDD UE release 13 and forward of category NB1.

This test case applies to all types of E-UTRA FDD UE release 14 and forward of category NB2.

This test case applies to all types of E-UTRA TDD UE release 15 and forward of category NB1 and NB2.

6.5.2.3F.3	Minimum conformance requirements

The category NB1 and NB2 UE relative in-band emission shall not exceed the values specified in Table 6.5.2.3F.3-1.

Table 6.5.2.3F.3-1: Minimum requirements for in-band emissions

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB |  |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -25 |  | Image frequencies (NOTES 2, 3) |
| Carrier leakage | dBc | -25 | 0 dBm ≤ Output power | Carrier frequency (NOTES 4, 5) |
|  |  | -20 | -30 dBm ≤ Output power ≤ 0 dBm |  |
|  |  | -10 | -40 dBm Output power < -30 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated tone. For each such tone, the minimum requirement is calculated as the higher of Ptone - 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. Ptone is defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 tone and the limit is expressed as a ratio of measured power in one non-allocated tone to the measured average power per allocated tone, where the averaging is done across all allocated tones. NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the centre carrier frequency, but excluding any allocated tones. NOTE 4: The measurement bandwidth is 1 tone and the limit is expressed as a ratio of measured power in one non-allocated tone to the measured total power in all allocated tones. NOTE 5: The applicable frequencies for this limit are those that are enclosed in the tones containing the DC frequency if is odd, or in the two tones immediately adjacent to the DC frequency if is even, but excluding any allocated tone. NOTE 6: is the Transmission Bandwidth (tones). NOTE 7: is the Transmission Bandwidth Configuration (tones). NOTE 8: is the starting frequency offset between the allocated tone and the measured non-allocated tone. (e.g. or for the first adjacent tone outside of the allocated bandwidth. NOTE 9: is the transmitted power per 3.75 kHz or 15 kHz in allocated tones, measured in dBm. |  |  |  |  |

The normative reference for this requirement is TS 36.101[2] clause 6.5.2F.3.

##### 6.5.2.3F.4	Test description

##### 6.5.2.3F.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions and test frequencies based on the subset of E-UTRA operating bands defined for NB-IoT in clause 5.2F. All of these configurations shall be tested with applicable test parameters shown in table 6.5.2.3F.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2.4. Configurations of NPDSCH and NPDCCH before measurement are specified in Annex C.2.

Table 6.5.2.3F.4.1-1: Test Configuration Table for FDD & TDD

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 8.1.1 |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 8.1.3.1 |  | Frequency ranges defined in Annex K.1.1 |  |  |
| Test Parameters |  |  |  |  |
| Configuration ID | Downlink Configuration | Uplink Configuration |  |  |
|  | N/A | Modulation | Ntones | Sub-carrier spacing (kHz) |
| 1 |  | QPSK | 1@0 | 3.75kHz |
| 2 |  | QPSK | 1@47 | 3.75kHz |
| 3 |  | QPSK | 1@0 | 15kHz |
| 4 |  | QPSK | 1@11 | 15kHz |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.3 using only main Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.4.0.

4.	The UL Reference Measurement channels are set according to Table 6.5.2.3F.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5.

##### 6.5.2.3F.4.2	Test procedure

1. 	SS sends uplink scheduling information via NPDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.5.2.3F.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Measure In-band emission using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test.

3.	Release the connection through State 3A-NB.

4.	Modify system information elements according to Table 6.5.2.3F.4.3-1 and Table 6.5.2.3F.4.3-2 and notify the UE via paging message with SystemInformationModification included.

5.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5 using the new UL power control setting.

6.	SS sends uplink scheduling information for UL HARQ process via NPDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.5.2.3F.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

7.	Measure In-band emission using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test.

NOTE 1:	For configuration IDs applicable to UE depending on UE capability in Test Configuration Table with different UL sub-carrier spacing, the SS shall release the connection through State 3A-NB and finally ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5 using the appropriate UL subcarrier spacing in Random Access Response message.

##### 6.5.2.3F.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 8.1.6 with the following exceptions.

Table 6.5.2.3F.4.3-1: P0-NominalNPUSCH-r13 configuration for test point 2

| Derivation Path: 36.508 clause 8.1.6.3 Table 8.1.6.3-14: UplinkPowerControlCommon-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| p0-NominalNPUSCH-r13 | -117 (dBm) |  |  |
| alpha-r13 | al1 (1) |  |  |
| deltaPreambleMsg3-r13 | 4 |  |  |
| } |  |  |  |

Table 6.5.2.3F.4.3-2: NPDSCH-ConfigCommon-NB-DEFAULT configuration for test point 2

| Derivation Path: 36.508 clause 8.1.6.3 Table 8.1.6.3-4: NPDSCH-ConfigCommon-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPDSCH-ConfigCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| nrs-Power-r13 | 21 (dBm) |  |  |
| } |  |  |  |

##### 6.5.2.3F.5	Test requirement

Each of the 20 In-band emissions results, derived in [Annex E.4.3] shall not exceed the corresponding values in Table 6.5.2.3F.5-1 based on supported UE release version.

Table 6.5.2.3F.5-1: Test requirements for in-band emissions for UE category NB1 and NB2

| Parameter description | Test point | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- | --- |
| General | 1, 2 | dB | +0.8 |  | Any non-allocated (NOTE 2) |
| IQ Image |  | dB | -24.2 |  | Image frequencies (NOTES 2, 3) |
| Carrier leakage | 1 | dBc | -24.2 | 0 dBm ≤ Output power f ≤ 3.0GHz: 3.2dBm ±3.2dB | Carrier frequency (NOTES 4, 5) |
|  | 2 |  | -19.2 | -30 dBm ≤ Output power ≤ 0 dBm f ≤ 3.0GHz: -26.8 dBm ±3.2dB |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated tone. For each such tone, the minimum requirement is calculated as the higher of Ptone - 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. Ptone is defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 tone and the limit is expressed as a ratio of measured power in one non-allocated tone to the measured average power per allocated tone, where the averaging is done across all allocated tones. NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the centre carrier frequency, but excluding any allocated tones. NOTE 4: The measurement bandwidth is 1 tone and the limit is expressed as a ratio of measured power in one non-allocated tone to the measured total power in all allocated tones. NOTE 5: The applicable frequencies for this limit are those that are enclosed in the tones containing the DC frequency if is odd, or in the two tones immediately adjacent to the DC frequency if is even, but excluding any allocated tone. NOTE 6: is the Transmission Bandwidth (tones). NOTE 7: is the Transmission Bandwidth Configuration (tones). NOTE 8: is the starting frequency offset between the allocated tone and the measured non-allocated tone. (e.g. or for the first adjacent tone outside of the allocated bandwidth. NOTE 9: is the transmitted power per 3.75 kHz or 15 kHz in allocated tones, measured in dBm. |  |  |  |  |  |

#### 6.5.2.3G	In-band emissions for non-allocated RB for V2X Communication

##### 6.5.2.3G.1	In-band emissions for non-allocated RB for V2X Communication / Non-concurrent with E-UTRA uplink transmissions

###### 6.5.2.3G.1.1	Test Purpose

The in-band emissions are a measure of the interference falling into the non-allocated resources blocks, this is to verify that the inband emissions of V2X sidelink non-concurrent with E-UTRAN uplink transmissions satisfy the minimum requirements listed in 6.5.2.3G.1.3.

###### 6.5.2.3G.1.2	Test applicability

This test case applies to all types of UE that supports V2X Sidelink communication and Band 47.

###### 6.5.2.3G.1.3	Minimum conformance requirements

The in-band emission is defined as the average across 12 sub-carrier and as a function of the RB offset from the edge of the allocated sidelink transmission bandwidth. The in-band emission is measured as the ratio of the UE output power in a non–allocated RB to the UE output power in an allocated RB.

The basic in-band emissions measurement interval is defined over one slot in the time domain When V2X transmissions are shortened due to transmission gap of 1 symbol at the end of the subframe, the In-band emissions measurement interval is reduced by one symbol, accordingly.

For V2X sidelink physical channels PSCCH, PSSCH and PSBCH, the In-band emissions requirements shall be as specified for PUSCH in subclause 36.101[2] 6.5.2.3 for the corresponding modulation and transmission bandwidth. For intra-band contiguous multi-carrier operation the in-band emission requirement of subclause 36.101 [2] 6.5.2.3 shall apply for each component carrier.

Consequently, the relative in-band emission of each sidelink physical channel shall not exceed the values specified in Tables 6.5.2.3G.1.3.

Table 6.5.2.3G.1.3-1: Minimum requirements for in-band emissions

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | For Frame Structure Type 1 and Frame Structure Type 2: For Frame Structure Type 3 and 20 MHz channel bandwidth: max-10-6∆RB-1,-57 dBm/180 kHz-PRB |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -28 | Image frequencies when carrier centre frequency < 1 GHz and Output power > 10 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -25 | Image frequencies when carrier centre frequency < 1 GHz and Output power ≤ 10 dBm |  |
|  |  | -25 | Image frequencies when carrier centre frequency ≥ 1 GHz |  |
| Carrier leakage | dBc | -28 | Output power > 10 dBm and carrier centre frequency < 1 GHz | Carrier frequency (NOTES 4, 5) |
|  |  | -25 | Output power > 10 dBm and carrier centre frequency ≥ 1 GHz |  |
|  |  | -25 | 0 dBm ≤ Output power ≤10 dBm |  |
|  |  | -20 | -30 dBm ≤ Output power ≤ 0 dBm |  |
|  |  | -10 | -40 dBm Output power < -30 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB - 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For Frame Structure Type 3 and 20 MHz channel bandwidth, the requirement applies with for any non-allocated RB andin the uplink scheduling grant with specified in [6]. NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the centre carrier frequency, but excluding any allocated RBs. For Frame Structure Type 3 with 20 MHz channel bandwidth, the applicable frequency is . NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. For Frame Structure Type 3 and 20 MHz channel bandwidth, the requirement applies within the uplink scheduling grant. NOTE 5: The applicable frequencies for this limit are those that are enclosed in the RBs containing the DC frequency if is odd, or in the two RBs immediately adjacent to the DC frequency if is even, but excluding any allocated RB. NOTE 6: is the Transmission Bandwidth (see Figure 5.6-1). NOTE 7: is the Transmission Bandwidth Configuration (see Figure 5.6-1). NOTE 8: is the limit specified in Table 6.5.2.1.1-1 for the modulation format used in the allocated RBs. NOTE 9: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. or for the first adjacent RB outside of the allocated bandwidth. NOTE 10: is the transmitted power per 180 kHz in allocated RBs, measured in dBm. [NOTE 11: For V2V-V2X waveforms, the requirements are applied when PSSCH and PSCCH are adjacent in frequency. The limit values (General, IQ Image or Carrier leakage) of each channel is calculated separately as Pgeneral, PSSCH, PIQ, PSSCH, PLO, PSSCH, Pgeneral, PSCCH, PIQ, PSCCH, PLO, PSCCH ,respectively. In-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB, PSSCH - 30 dB and the PsumIBE.SSCH, where PsumIBE.SSCH is the linear sum of Pgeneral, PSSCH, PIQ, PSSCH, PLO, PSSCH, Pgeneral, PSCCH, PIQ, PSCCH, PLO, PSCCH.] NOTE12: The requirement in this power range is not applicable for V2X sidelink transmission for E-UTRA V2X operating bands specified in Table 5.5G-1. |  |  |  |  |

The normative reference for this requirement is TS 36.101 [2] clause 6.5.2G.3.

###### 6.5.2.3G.1.4	Test description

6.5.2.3G.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in sub-clause 5.2G. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.5.2.3G.1.4.1-1 and 6.5.2.3G.1.4.1-2. The details of the V2X reference measurement channels (RMCs) are specified in sub-clause A.8.3 and the GNSS configuration in TS 36.508 [7] subclause 4.11.

Table 6.5.2.3G.1.4.1-1: Test Configuration Table for PSSCH and PSCCH

| Initial Conditions |  |  |
| --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  | NC |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  | Low range, Mid range, High range |
| Test Channel Bandwidths (as specified in subclause 5.4.2G) |  | Lowest, Highest |
| Test Parameters for Channel Bandwidths |  |  |
|  | V2X Configuration to Transmit |  |
| Ch BW | Mod'n | PSSCH RB Allocation (Note 2) |
| 10MHz | QPSK | 12 |
| 20MHz | QPSK | 18 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in specified in subclause 5.4.2G. Note 2: The test shall be performed with allocated sub-channel(s) at low and high end of the channel bandwidth respectively. |  |  |

Table 6.5.2.3G.1.4.1-2: Test Configuration Table for PSBCH

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  | NC |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths (as specified in subclause 5.4.2G) |  | Lowest, Highest |  |
| Test parameters for channel bandwidths |  |  |  |
| PSBCH Configuration |  |  |  |
| Ch BW | Mod’n |  | RB allocation |
| 10MHz | QPSK |  | 6 |
| 20MHz | QPSK |  | 6 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in subclause 5.4.2G. |  |  |  |

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 36.508[7] Figure A.92.

2.	The parameter settings for the V2X sidelink transmission over PC5 for both SS and UE are pre-configured according to TS 36.508 [7] subclause 4.10.1 with the exception specified in clause 6.5.2.3G.1.4.3.

3.	The V2X reference measurement channel is set according to Table 6.5.2.3G.1.4.1-1

4.	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS36.508 [7] Table 4.11.2-2. Geographical area #1 is also pre-configured in the UE.

5.	Propagation conditions are set according to Annex B.0;

6.	Ensure the UE is in Side link State 5A-V2X in Transmit Mode, according to TS 36.508 [7] clause 4.5.9;

7.	The GNSS simulator is triggered to start step 1 of Scenario #1 to simulate a location in the centre of Geographical area #1. Wait for the UE to acquire the GNSS signal and start to transmit.

6.5.2.3G.1.4.2	Test procedure

Test procedure for PSCCH+PSSCH:

[TP1: V2X UE output power > 0dBm]

1.	Set the V2X sidelink communication parameters for both the V2X sidelink capable UE (hereinafter referred to as V2X UE) and SS according to SL-V2X-Preconfiguration in TS36.508[7] subclause 4.10.1 with the exception specified in table 6.5.2.3G.1.4.3-1;

2.	V2X UE schedules the V2X RMC according to SL-V2X-Preconfiguration which is in line with the test configuration in Table 6.5.2.3G.1.4.1-1;

3.	Measure the V2X UE output power to make sure V2X UE transmission power to be 4.3dBm+/-4.3dB for carrier frequency f > 5GHz;

4.	Measure In-band emission using Global In-Channel Tx-Test (Annex E)

[TP2: V2X UE output power within (0 -30) dBm]

Repeat the above steps 1~4 with the exception that making sure V2X UE transmission power to be -25.5dBm+/-4.5dB for carrier frequency f > 5GHz in step3.

Test procedure for PSBCH

[TP1: V2X UE output power > 0dBm]

1.	Set the V2X sidelink communication parameters for both the V2X sidelink capable UE (hereinafter referred to as V2X UE) and SS according to SL-V2X-Preconfiguration in TS36.508[7] subclause 4.10.1 with the exception specified in table 6.5.2.3G.1.4.3-3;

2.	V2X UE schedules the SLSS/PSBCH according to SL-V2X-Preconfiguration which is in line with the test configuration in Table 6.5.2.3G.1.4.1-2;

3.	Measure the V2X UE output power to make sure V2X UE transmission power to be 4.3dBm+/-4.3dB for carrier frequency  f > 5GHz;

4.	Measure In-band emission using Global In-Channel Tx-Test (Annex E)

[TP2: V2X UE output power within (0 -30) dBm]

Repeat the above steps 1~4 with the exception that making sure V2X UE transmission power to be -25.5dBm+/-4.5 dB for carrier frequency f > 5GHz in step3.

6.5.2.3G.1.4.3	Message contents

SL-V2X-Preconfiguration is according to TS 36.508 [7] Table 4.10.1.1-1 with the exceptions specified in Table 6.5.2.3G.1.4.3-1, Table 6.5.2.3G.1.4.3-2 and Table 6.5.2.3G.1.4.3-3:

Table 6.5.2.3G.1.4.3-1: SL-V2X-PreconfigFreqInfo-r14-DEFAULT

| Derivation Path: 36.508 table 4.6.3-20K |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-V2X-PreconfigFreqInfo-r14-DEFAULT ::= SEQUENCE { |  |  |  |
| v2x-CommPreconfigGeneral-r14 SEQUENCE { |  | SL-PreconfigGeneral-r12 |  |
| maxTxPower-r12 | 5 |  | PSSCH+PSCCH TP1 |
|  | -25 |  | PSSCH+PSCCH TP2, |
| sl-bandwidth-r12 | n50 | 10Mhz |  |
|  | n100 | 20Mhz |  |
| } |  |  |  |
| } |  |  |  |
| v2x-CommPreconfigSync-r14 SEQUENCE { |  | SL-PreconfigSync-r12 |  |
| syncOffsetIndicators-r14 SEQUENCE { |  |  |  |
| syncOffsetIndicator1-r14 | Not Present | Double synchronization signal transmission |  |
| } |  |  |  |
| } |  |  |  |
| v2x-CommRxPoolList-r14 SEQUENCE (SIZE (1..maxSL-V2X-RxPoolPreconf-r14)) OF SL-V2X-PreconfigCommPool-r14 { | 1 entry |  |  |
| SL-V2X-PreconfigCommPool-r14[1] | SL-V2X-PreconfigCommPool-r14-DEFAULT using condition BITMAP_6 and exception listed in Table Table 6.5.2.3G.1.4.3-2 |  |  |
| } |  |  |  |
| v2x-CommTxPoolList-r14 SEQUENCE (SIZE (1..maxSL-V2X-TxPoolPreconf-r14)) OF SL-V2X-PreconfigCommPool-r14 { |  |  |  |
| SL-V2X-PreconfigCommPool-r14[1] | SL-V2X-PreconfigCommPool-r14-DEFAULT using condition BITMAP_6 and exception listed in Table 6.5.2.3G.1.4.3-2 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.2.3G.1.4.3-2: SL-V2X-PreconfigCommPool-r14-DEFAULT

| Derivation Path: 36.508 clause Table 4.6.3-20J |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-CommResroucePoolV2X-r14-DEFAULT ::= SEQUENCE { |  |  |  |
| sl-Subframe-r14 CHOICE { |  |  |  |
| bs20-r14 | 11111111111111111111 |  | BITMAP_6 |
| } |  |  |  |
| startRB-Subchannel-r14 | As required in the test configuration tables in subclause 6.5.2.3G.1.4.1 | The starting resource block shall be RB #0 and RB# (max+1 - RB allocation) of the channel bandwidth. |  |
| dataTxParameters-r14 SEQUENCE { |  |  |  |
| p0-r12 | 31 | It’s set to the maximum value to disable its effect on transmission power. |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.2.3G.1.4.3-3: SL-V2X-PreconfigFreqInfo-r14-DEFAULT for PSBCH test

| Derivation Path: 36.508 table 4.6.3-20K |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-V2X-PreconfigFreqInfo-r14-DEFAULT ::= SEQUENCE { |  |  |  |
| v2x-CommPreconfigGeneral-r14 SEQUENCE { |  | SL-PreconfigGeneral-r12 |  |
|  | 5 |  | PSBCH TP1 |
|  | -25 |  | PSBCH TP2, |
| sl-bandwidth-r12 | n50 | 10Mhz |  |
|  | n100 | 20Mhz |  |
| } |  |  |  |
| v2x-CommPreconfigSync-r14 SEQUENCE { |  | SL-PreconfigSync-r12 |  |
| syncOffsetIndicators-r14 SEQUENCE { |  |  |  |
| syncOffsetIndicator1-r14 | 0 |  |  |
| syncOffsetIndicator2-r14 | 1 |  |  |
| } |  |  |  |
| syncTxParameters-r14 | 31 | It’s set to the maximum value to disable its effect on transmission power. |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| BW10 | 10 MHz channel bandwidth cell environment |
| BW20 | 20 MHz channel bandwidth cell environment |
| PSSCH+PSCCH TPn | Test point n for PSCCH+PSSCH defined in the section of test procedure, n = 1,2 |
| PSBCH TPn | Test point n for PSBCH defined in the section of test procedure, n = 1,2 |

###### 6.5.2.3G.1.5	Test requirement

Each of the 20 In-band emissions results, derived in Annex E.4.3 shall not exceed the corresponding values in Tables 6.5.2.3G.1.5-1.

Table 6.5.2.3G.1.5-1: Test requirements for in-band emissions for UE in V2X sidelink communication

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | For Frame Structure Type 1 and Frame Structure Type 2: +0.8 |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -24.2 | Image frequencies when carrier centre frequency ≥ 1 GHz | Image frequencies (NOTES 2, 3) |
| Carrier leakage |  | -24.2 | Output power >10 dBm | Carrier frequency (NOTE 4,5) |
|  |  | -19.2 | -30 dBm ≤ Output power ≤ 0 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB - 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For Frame Structure Type 3 and 20 MHz channel bandwidth, the requirement applies with for any non-allocated RB andin the uplink scheduling grant with specified in [6]. NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the centre carrier frequency, but excluding any allocated RBs. For Frame Structure Type 3 with 20 MHz channel bandwidth, the applicable frequency is . NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. For Frame Structure Type 3 and 20 MHz channel bandwidth, the requirement applies within the uplink scheduling grant. NOTE 5: The applicable frequencies for this limit are those that are enclosed in the RBs containing the DC frequency if is odd, or in the two RBs immediately adjacent to the DC frequency if is even, but excluding any allocated RB. NOTE 6: is the Transmission Bandwidth (see Figure 5.6-1). NOTE 7: is the Transmission Bandwidth Configuration (see Figure 5.6-1). NOTE 8: is the limit specified in Table 6.5.2.1.1-1 for the modulation format used in the allocated RBs. NOTE 9: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. or for the first adjacent RB outside of the allocated bandwidth. NOTE 10: is the transmitted power per 180 kHz in allocated RBs, measured in dBm. [NOTE 11: For V2V-V2X waveforms, the requirements are applied when PSSCH and PSCCH are adjacent in frequency. The limit values (General, IQ Image or Carrier leakage) of each channel is calculated separately as Pgeneral, PSSCH, PIQ, PSSCH, PLO, PSSCH, Pgeneral, PSCCH, PIQ, PSCCH, PLO, PSCCH ,respectively. In-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB, PSSCH - 30 dB and the PsumIBE.SSCH, where PsumIBE.SSCH is the linear sum of Pgeneral, PSSCH, PIQ, PSSCH, PLO, PSSCH, Pgeneral, PSCCH, PIQ, PSCCH, PLO, PSCCH.] NOTE12: The requirement in this power range is not applicable for V2X sidelink transmission for E-UTRA V2X operating bands specified in Table 5.5G-1. |  |  |  |  |

##### 6.5.2.3G.2	In-band emissions for non-allocated RB for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions

6.5.2.3G.2.1	Test purpose

The in-band emissions are a measure of the interference falling into the non-allocated resources blocks, this is to verify that the inband emissions of V2X sidelink communication simultaneous with E-UTRA uplink/downlink satisfy the minimum requirements listed in 6.5.2.3G.2.3.

6.5.2.3G.2.2	Test Applicability

This test case applies to all types of E-UTRA UE release 14 and forward that supports V2X communication and is capable of V2X sidelink communication simultaneous with E-UTRA uplink/downlink.

6.5.2.3G.2.3	Minimum conformance requirements

When UE is configured for simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions for inter-band E-UTRA V2X / E-UTRA bands specified in 36.101[2]  Table 5.5G-2, the requirements in 36.101[2] subclause 6.5.2G apply for V2X sidelink transmission and the requirements in 36.101[2] subclause 6.5.2 apply for the E-UTRA uplink transmission.

For V2X sidelink physical channels PSCCH, PSSCH and PSBCH, the In-band emissions requirements shall be as specified for PUSCH in 36.101[2] subclause 6.5.2.3 for the corresponding modulation and transmission bandwidth. When V2X transmissions are shortened due to transmission gap of 1 symbol at the end of the subframe, the In-band emissions measurement interval is reduced by one symbol, accordingly.

Consequently, the relative in-band emission of each sidelink physical channel shall not exceed the values specified in Tables 6.5.2.3G.2.3-1.

Table 6.5.2.3G.2.3-1: Minimum requirements for in-band emissions

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | For Frame Structure Type 1 and Frame Structure Type 2: For Frame Structure Type 3 and 20 MHz channel bandwidth: max-10-6∆RB-1,-57 dBm/180 kHz-PRB |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -28 | Image frequencies when carrier centre frequency < 1 GHz and Output power > 10 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -25 | Image frequencies when carrier centre frequency < 1 GHz and Output power ≤ 10 dBm |  |
|  |  | -25 | Image frequencies when carrier centre frequency ≥ 1 GHz |  |
| Carrier leakage | dBc | -28 | Output power > 10 dBm and carrier centre frequency < 1 GHz | Carrier frequency (NOTES 4, 5) |
|  |  | -25 | Output power > 10 dBm and carrier centre frequency ≥ 1 GHz |  |
|  |  | -25 | 0 dBm ≤ Output power ≤10 dBm |  |
|  |  | -20 | -30 dBm ≤ Output power ≤ 0 dBm |  |
|  |  | -10 | -40 dBm Output power < -30 dBm(NOTE 12) |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB - 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For Frame Structure Type 3 and 20 MHz channel bandwidth, the requirement applies with for any non-allocated RB andin the uplink scheduling grant with specified in [6]. NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the centre carrier frequency, but excluding any allocated RBs. For Frame Structure Type 3 with 20 MHz channel bandwidth, the applicable frequency is . NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. For Frame Structure Type 3 and 20 MHz channel bandwidth, the requirement applies within the uplink scheduling grant. NOTE 5: The applicable frequencies for this limit are those that are enclosed in the RBs containing the DC frequency if is odd, or in the two RBs immediately adjacent to the DC frequency if is even, but excluding any allocated RB. NOTE 6: is the Transmission Bandwidth (see Figure 5.6-1). NOTE 7: is the Transmission Bandwidth Configuration (see Figure 5.6-1). NOTE 8: is the limit specified in Table 6.5.2.1.1-1 for the modulation format used in the allocated RBs. NOTE 9: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. or for the first adjacent RB outside of the allocated bandwidth. NOTE 10: is the transmitted power per 180 kHz in allocated RBs, measured in dBm. [NOTE 11: For V2V-V2X waveforms, the requirements are applied when PSSCH and PSCCH are adjacent in frequency. The limit values (General, IQ Image or Carrier leakage) of each channel is calculated separately as Pgeneral, PSSCH, PIQ, PSSCH, PLO, PSSCH, Pgeneral, PSCCH, PIQ, PSCCH, PLO, PSCCH ,respectively. In-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB, PSSCH - 30 dB and the PsumIBE.SSCH, where PsumIBE.SSCH is the linear sum of Pgeneral, PSSCH, PIQ, PSSCH, PLO, PSSCH, Pgeneral, PSCCH, PIQ, PSCCH, PLO, PSCCH.] NOTE 12 The requirement in this power range is not applicable for V2X sidelink transmission for E-UTRA V2X operating bands specified in Table 5.5G-1. |  |  |  |  |

The normative reference for this requirement is TS 36.101 [2] clause6.5.2G and 6.5.2G.3.

6.5.2.3G.2.4	Test description

6.5.2.3G.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on Inter-band con-current V2X configurations specified in table 5.4.2G.1-2. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.5.2.3G.2.4.1-1 and 6.5.2.3G.2.4.1-2. The details of the uplink and V2X reference measurement channels (RMCs) are specified in sub-clause A.2 and A.8.3 respectively. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.2.3G.2.4.1-1: Test Configuration Table for PSSCH and PSCCH

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range for Uplink and V2X Sidelink High range for Uplink and V2X Sidelink |  |  |  |
| Test Channel Bandwidths as specified in 5.4.2G |  |  | Lowest NRB_agg, Highest NRB_agg |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
| E-UTRA | Downlink | Uplink Configuration |  | V2X Configuration PSSCH and PSCCH |  |  |
| Ch BW | N/A for In-band emission test case | Mod'n | RB allocation (Note 2) | Ch BW | Mod'n | RB allocation (Note 4) |
| 1.4 MHz |  | QPSK | 1 | 10 MHz | QPSK | 12 |
| 3 MHz |  | QPSK | 4 | 20 MHz | QPSK | 18 |
| 5 MHz |  | QPSK | 8 |  |  |  |
| 10 MHz |  | QPSK | 12 |  |  |  |
| 15 MHz |  | QPSK | 16 |  |  |  |
| 20 MHz |  | QPSK | 18 |  |  |  |
| Note 1: V2X con-current band configurations are checked separately for each band configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2G.1-2. Note 2: The starting resource block for E-UTRA carrier and V2X carrier shall be RB #0+RB #0 and RB# (max+1 - RB allocation)+RB# (max+1 - RB allocation) of the channel bandwidth. Note 3: The Inter-band con-current V2X configurations and bandwidth combination sets in Table 5.4.2G.1-2, which UE supports, are tested across the uplink and v2x sidelink allocation combinations as above. Note 4: The test shall be performed with allocated sub-channel(s) at low and high end of the channel bandwidth respectively. |  |  |  |  |  |  |

Table 6.5.2.3G.2.4.1-2: Test Configuration Table for PSBCH

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range for Uplink and V2X Sidelink High range for Uplink and V2X Sidelink |  |  |  |
| Test Channel Bandwidths as specified in 5.4.2G |  |  | Lowest NRB_agg, Highest NRB_agg |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  | Downlink | Uplink Configuration |  | V2X Configuration PSBCH |  |  |
| Ch BW | N/A for In-band emission test case | Mod'n | RB allocation | Ch BW | Mod'n | RB allocation |
| 1.4 MHz |  | QPSK | 1 | 10 MHz | QPSK | 6 |
| 3 MHz |  | QPSK | 4 | 20 MHz | QPSK | 6 |
| 5 MHz |  | QPSK | 8 |  |  |  |
| 10 MHz |  | QPSK | 12 |  |  |  |
| 15 MHz |  | QPSK | 16 |  |  |  |
| 20 MHz |  | QPSK | 18 |  |  |  |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2G.1-2. Note 2: The starting resource block for E-UTRA carrier shall be RB #0 and RB# (max+1 - RB allocation) of the channel bandwidth. Note 3: The Inter-band con-current V2X configurations and bandwidth combination sets in Table 5.4.2G.1-2, which UE supports, are tested across the uplink and v2x sidelink allocation combinations as above |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.93a.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3. Message content exceptions are defined in clause 6.5.2.3G.2.4.3

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL and V2X Reference Measurement Channels are set according to subclause 6.5.2.3G.2.4.1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in the state 3A-RF-V2X according to TS 36.508 [7] clause 5.2A.2C.

6.5.2.3G.2.4.2	Test procedure

Test procedure for PSCCH+PSSCH:

[TP1: V2X UE output power > 10dBm]

1.	The SS configure the PSCCH and PSSCH transmission at the PCMAX,c,V2X level of the corresponding test point to enable V2X sidelink transmission power at 4.3dBm+/-4.3dB for carrier frequency  f > 5GHz.2.	The SS simultaneously sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC and V2X scheduling information via PDCCH DCI format 5A for SL_V_RNTI to schedule the V2X RMC according to Table 6.5.2.3G.2.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL and V2X RMCs.

3.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE E-UTRA uplink output power is 3.2 dBm+/-3.2dB for carrier frequency f ≤ 3.0GHz or 13.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

4.	Measure In-band emission using Global In-Channel Tx-Test (Annex E) on all the uplink and V2X sidelink carriers with PRB allocated. On E-UTRA carrier, the TDD slots with transient periods are not under test.

[TP2: V2X UE output power within (0,-30) dBm]

Repeat the above steps 1~4 with the exception in step3 that making sure UE E-UTRA uplink output power is -26.8 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -26.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz  and the exception in step 1 that V2X sidelink transmission power is -25.5dBm+/-4.5 dB for carrier frequency  f > 5GHz.

Test procedure for PSBCH

[TP1: V2X UE output power > 0dBm]

1.	The SS configures PSBCH transmission at the PCMAX,c,V2X level of the corresponding test point to enable V2X sidelink transmission power at 4.3dBm+/-4.3dB for carrier frequency  f > 5GHz according to table 6.5.2.3G.2.4.3-3.

2.	The SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC, Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL.

3.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE E-UTRA uplink output power is 3.2 dBm+/-3.2dB for carrier frequency f ≤ 3.0GHz or 3.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz

4.	Measure In-band emission using Global In-Channel Tx-Test (Annex E) on the uplink and V2X sidelink carrier with PRB allocated. On E-UTRA carrier, the TDD slots with transient periods are not under test.

[TP2: V2X UE output power within (0,-30) dBm]

Repeat the above steps 1~4 with the exception in step 3 that making sure UE E-UTRA uplink output power is -26.8 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -26.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz and the exception in step 1 that V2X UE transmission power is -25.5dBm+/-4.5 dB for carrier frequency f > 5GHz.

6.5.2.3G.2.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the following exceptions:

Table 6.5.2.3G.2.4.3-1: SL-PSSCH-TxConfig-r14-DEFAULT

| Derivation Path: TS 36.508 [7] clause 4.6, Table 4.6.3-20F SL-PSSCH-TxConfig-r14-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| maxTxPower-r14 | 0 |  | PSSCH+PSCCH TP1 |
|  | -30 |  | PSSCH+PSCCH TP2 |

Table 6.5.2.3G.2.4.3-2: SL-CommResourcePoolV2X-r14-DEFAULT

| Derivation Path: TS 36.508 [7] clause 4.6, Table 4.6.3-20C: SL-CommResourcePoolV2X-r14-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| dataTxParameters-r14 SEQUENCE { |  |  |  |
| alpha-r12 | a0 |  |  |
| p0-r12 | 31 |  |  |
| } |  |  |  |

Table 6.5.2.3G.2.4.3-3: RRCConnectionReconfiguration (SideLink) for PSBCH test

| Derivation Path: 36.508 table 4.6.1-8B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sl-SyncTxControl-r12 SEQUENCE { |  |  |  |
| networkControlledSyncTx-r12 | on | This field indicates the UE shall transmit synchronisation information |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension { |  |  |  |
| sl-V2X-ConfigDedicated-r14 SEQUENCE { |  |  |  |
| v2x-InterFreqInfoList-r14 SEQUENCE (SIZE (0..maxFreqV2X-1-r14)) OF SL-InterFreqInfoV2X-r14 { |  |  |  |
| sl-MaxTxPower-r14 | 0 |  | PSBCH TP1 |
|  | -30 |  | PSBCH TP2 |
| v2x-UE-ConfigList-r14 SEQUENCE (SIZE (1.. maxCellIntra)) OF SL-V2X-InterFreqUE-Config-r14 { |  |  |  |
| v2x-SyncConfig-r14 SEQUENCE (SIZE (1..maxSL-V2X-SyncConfig-r14)) OF SL-SyncConfigNFreq-r13 { |  |  |  |
| txParameters-r13 SEQUENCE { |  |  |  |
| syncTxParameters-r13 SEQUENCE { |  |  |  |
| alpha-r12 | 0 |  |  |
| p0-r12 | 31 |  |  |
| } |  |  |  |
| syncTxPeriodic-r13 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.5.2.3G.2.5	Test requirement

For E-UTRA uplink and V2X sidelink, each of the In-band emissions results, derived in Annex E.4.3 shall not exceed the corresponding values in Tables 6.5.2.3G.2.5-1.

Table 6.5.2.3G.2.5-1: Test requirements for in-band emissions for UE in V2X sidelink communication

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | For Frame Structure Type 1 and Frame Structure Type 2: +0.8 For Frame Structure Type 3 and 20 MHz channel bandwidth: max-10-6∆RB-1,-57 dBm/180 kHz-PRB |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -27.2 | Image frequencies when carrier centre frequency < 1 GHz and Output power > 10 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -24.2 | Image frequencies when carrier centre frequency < 1 GHz and Output power ≤ 10 dBm |  |
|  |  | -24.2 | Image frequencies when carrier centre frequency ≥ 1 GHz |  |
| Carrier leakage | dBc | -27.2 | Output power > 10 dBm and carrier centre frequency < 1 GHz | Carrier frequency (NOTES 4, 5) |
|  |  | -24.2 | Output power > 10 dBm and carrier centre frequency ≥ 1 GHz |  |
|  |  | -24.2 | 0 dBm ≤ Output power ≤10 dBm |  |
|  |  | -19.2 | -30 dBm ≤ Output power ≤ 0 dBm |  |
|  |  | -9.2 | -40 dBm Output power < -30 dBm(NOTE 12) |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB - 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For Frame Structure Type 3 and 20 MHz channel bandwidth, the requirement applies with for any non-allocated RB andin the uplink scheduling grant with specified in [6]. NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the centre carrier frequency, but excluding any allocated RBs. For Frame Structure Type 3 with 20 MHz channel bandwidth, the applicable frequency is . NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. For Frame Structure Type 3 and 20 MHz channel bandwidth, the requirement applies within the uplink scheduling grant. NOTE 5: The applicable frequencies for this limit are those that are enclosed in the RBs containing the DC frequency if is odd, or in the two RBs immediately adjacent to the DC frequency if is even, but excluding any allocated RB. NOTE 6: is the Transmission Bandwidth (see Figure 5.6-1). NOTE 7: is the Transmission Bandwidth Configuration (see Figure 5.6-1). NOTE 8: is the limit specified in Table 6.5.2.1.1-1 for the modulation format used in the allocated RBs. NOTE 9: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. or for the first adjacent RB outside of the allocated bandwidth. NOTE 10: is the transmitted power per 180 kHz in allocated RBs, measured in dBm. [NOTE 11: For V2V-V2X waveforms, the requirements are applied when PSSCH and PSCCH are adjacent in frequency. The limit values (General, IQ Image or Carrier leakage) of each channel is calculated separately as Pgeneral, PSSCH, PIQ, PSSCH, PLO, PSSCH, Pgeneral, PSCCH, PIQ, PSCCH, PLO, PSCCH ,respectively. In-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB, PSSCH - 30 dB and the PsumIBE.SSCH, where PsumIBE.SSCH is the linear sum of Pgeneral, PSSCH, PIQ, PSSCH, PLO, PSSCH, Pgeneral, PSCCH, PIQ, PSCCH, PLO, PSCCH.] NOTE 12: The requirement in this power range is not applicable for V2X sidelink transmission for E-UTRA V2X operating bands specified in Table 5.5G-1. |  |  |  |  |

##### 6.5.2.3G.3	In-band emissions for non-allocated RB for V2X Communication / Intra-band contiguous MCC operation

6.5.2.3G.3.1	Test purpose

The in-band emissions are a measure of the interference falling into the non-allocated resources blocks, this is to verify that the inband emissions of V2X sidelink with intra-band contiguous MCC satisfy the minimum requirements listed in 6.5.2.3G.3.3.

##### 6.5.2.3G.3.2	Test Applicability

This test case applies to all types of UE that supports Intra-band contiguous multi-carrier operation V2X Sidelink communication and Band 47.

##### 6.5.2.3G.3.3	Minimum conformance requirements

For intra-band contiguous carrier aggregation bandwidth class B, the requirements in Table 6.5.2.3G.3.3-1 and 6.5.2.3G.3.3-2 apply within the aggregated transmission bandwidth configuration with both component carrier (s) active and one single contiguous PRB allocation of bandwidth  at the edge of the aggregated transmission bandwidth configuration.

Table 6.5.2.3G.3.3-1: Minimum requirements for in-band emissions (allocated component carrier)

| Parameter | Unit | Limit |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB |  |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -25 |  | Exception for IQ image (NOTE 3) |
| Carrier leakage | dBc | -25 | Output power > 0 dBm | Exception for Carrier frequency (NOTE 4) |
|  |  | -20 | -30 dBm ≤ Output power ≤ 0 dBm |  |
|  |  | -10 | -40 dBm Output power < -30 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB - 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in NOTE 9. The limit is evaluated in each non-allocated RB. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs NOTE 3: Exceptions to the general limit are allowed for up to +1 RBs within a contiguous width of +1 non-allocated RBs. The measurement bandwidth is 1 RB. NOTE 4: Exceptions to the general limit are allowed for up to two contiguous non-allocated RBs. The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in the non-allocated RB to the measured total power in all allocated RBs. NOTE 5: is the Transmission Bandwidth (see Figure 5.4.2-1) not exceeding NOTE 6: is the Transmission Bandwidth Configuration (see Figure 5.4.2-1) of the component carrier with RBs allocated. NOTE 7: is the limit specified in Table 6.5.2.1G.3-1 for the modulation format used in the allocated RBs. NOTE 8: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. or for the first adjacent RB outside of the allocated bandwidth). NOTE 9: is the transmitted power per 180 kHz in allocated RBs, measured in dBm. NOTE 10: For V2X intra-band contiguous multi-carrier operation the in-band emission requirement (General, IQ Image or Carrier leakage) of each channel is calculated separately as Pgeneral, PSSCH,c(i), PIQ, PSSCH,c(i), PLO, PSSCH,c(i), Pgeneral, PSCCH,c(i), PIQ, PSCCH,c(i), PLO, PSCCH,c(i) for each carrier respectively. In-band emissions combined limit for the aggregated bandwidth is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the PsumIBE.CA, where PsumIBE.CA is the linear sum of Pgeneral, PSSCH,c(i), PIQ, PSSCH,c(i), PLO, PSSCH,c(i), Pgeneral, PSCCH,c(i), PIQ, PSCCH,c(i), PLO, PSCCH,c(i) for all carriers. |  |  |  |  |

Table 6.5.2.3G.3.3-2: Minimum requirements for in-band emissions (not allocated component carrier)

| Para-meter | Unit | Meas BW NOTE 1 | Limit |  | remark | Applicable Frequencies |
| --- | --- | --- | --- | --- | --- | --- |
| General | dB | BW of 1 RB (180KHz rectangular) |  |  | The reference value is the average power per allocated RB in the allocated component carrier | Any RB in the non allocated component carrier. The frequency raster of the RBs is derived when this component carrier is allocated with RBs |
| IQ Image | dB | BW of 1 RB (180KHz rectangular) | -25 (NOTE 2) |  | The reference value is the average power per allocated RB in the allocated component carrier | The frequencies of the contiguous non-allocated RBs are unknown. The frequency raster of the RBs is derived when this component carrier is allocated with RBs |
| Carrier leakage | dBc | BW of 1 RB (180KHz rectangular) | NOTE 3 |  | The reference value is the total power of the allocated RBs in the allocated component carrier | The frequencies of the up to 2 non-allocated RBs are unknown. The frequency raster of the RBs is derived when this component carrier is allocated with RBs |
|  |  |  | -25 | Output power > 0 dBm |  |  |
|  |  |  | -20 | -30 dBm ≤ Output power ≤ 0 dBm |  |  |
|  |  |  | -10 | -40 dBm Output power < -30 dBm |  |  |
| NOTE1: Resolution BWs smaller than the measurement BW may be integrated to achieve the measurement bandwidth. NOTE 2: Exceptions to the general limit are allowed for up to +1 RBs within a contiguous width of +1 non-allocated RBs. NOTE 3: Two Exceptions to the general limit are allowed for up to two contiguous non-allocated RBs NOTE 4: NOTES 1, 5, 6, 7, 8, 9 from Table 6.5.2.3G.3.3-1 apply for Table 6.5.2.3G.3.3-2 as well. NOTE 5: for measured non-allocated RB in the non allocated component carrier may take non-integer values when the carrier spacing between the CCs is not a multiple of RB. NOTE 6: For V2X intra-band contiguous multi-carrier operation the in-band emission requirement (General, IQ Image or Carrier leakage) of each channel is calculated separately as Pgeneral, PSSCH,c(i), PIQ, PSSCH,c(i), PLO, PSSCH,c(i), Pgeneral, PSCCH,c(i), PIQ, PSCCH,c(i), PLO, PSCCH,c(i) for each carrier respectively. In-band emissions combined limit for the aggregated bandwidth is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the PsumIBE.CA, where PsumIBE.CA is the linear sum of Pgeneral, PSSCH,c(i), PIQ, PSSCH,c(i), PLO, PSSCH,c(i), Pgeneral, PSCCH,c(i), PIQ, PSCCH,c(i), PLO, PSCCH,c(i) for all carriers. |  |  |  |  |  |  |

The normative reference for this requirement is TS 36.101 [2] subclause 6.5.2G.3 and subclause 6.5.2A.3

6.5.2.3G.3.4	Test Description

6.5.2.3G.3.4.1	Initial Conditions

Initial conditions specify the set of UE parameters that UE needs be test in and the steps that SS should take with UE to reach the correct measurement state.

The initial test conditions consist of environment conditions, test frequencies and channel bandwidth based on E-UTRAN operating bands specified in table 5.4.2G.1-4 and propagation conditions. All these configurations shall be tested with applicable parameters for each channel bandwidth, and are shown in table 6.5.2.3G.3.4.1-1, the details of reference channel are specified in sub-clause A.8.3 and the reference GNSS configuration in TS 36.508 [7] subclause 4.11.

6.5.2.3G.3.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, High range |  |
| Test CC Combination setting(NRB_agg) as specified in subclause 5.4.2G |  |  | Lowest NRB_agg, Highest NRB_agg |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| V2X Multi-carrier Configuration |  | V2X PSSCH Allocation |  |  |
| CC1 NRB | CC2 NRB | CC1 & CC2 Mod |  | CC1 and CC2 RB Allocation |
| 50 | 50 | QPSK |  | 12@2+0@0 |
| 50 | 50 | QPSk |  | 0@0+12@37 |

6.5.2.3G.3.4.1-2: Table Initial Conditions for PSBCH

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | NC |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  |  | Low range, Mid range , High range |
| Test CC Combination setting(NRB_agg) as specified in subclause 5.4.2G |  |  |  | Lowest NRB_agg, Highest NRB_agg |
| Test Parameters for V2X MCC Configuration |  |  |  |  |
| V2X MCC Channel Bandwidth |  |  | V2X Sidelink PSBCH Configuration |  |
| Configuration ID | CC1 NRB | CC2 NRB | CC1 & CC2 Mod | RB allocation |
| 1 | 50 | 50 | QPSK | 6 |

1. Connect the SS and GNSS simulator to UE antenna connectors according to the figure A.89a in TS 36.508 [7] Annex A.

1a. The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS36.508 [7] Table 4.11.2-2. Geographical area #1 is also pre-configured in the UE.

2. Propagation conditions are set according to Annex B.0.

3.	Trigger the UE to reset UTC time. (NOTE: The UTC time reset may be performed by MMI or AT command (+CUTCR).)

4.	The GNSS simulator is triggered to start step 1 of Scenario #1 to simulate a location in the centre of Geographical area #1. Wait for the UE to acquire the GNSS signal and start to transmit.

6.5.2.3G.3.4.2	Test Procedure

Test procedure for PSCCH+PSSCH:

[TP1: V2X UE output power > 0dBm]

1.	Set the V2X sidelink communication parameters for both the V2X sidelink capable UE (hereinafter referred to as V2X UE) and SS according to SL-V2X-Preconfiguration in TS 36.508[7] subclause 4.10.1 with the exception specified in clause 6.5.2.3G.3.4.3-1;

2.	Ensure the UE is in the Side link State 5A-V2X in Transmit Mode, according to TS 36.508 [7] clause 4.5.9. Message contents are defined in clause 6.5.2.3G.3.4.3

3.	V2X UE schedules the V2X RMC according to SL-V2X-Preconfiguration in line with the test configuration in Table 6.5.2.3G.3.4.1-1;

4.	Measure the V2X UE output power to make sure V2X UE transmission power to be 4.3dBm+/-4.3dB for carrier frequency  f > 5GHz;

5.	Measure In-band emission using Global In-Channel Tx-Test (Annex E) on the component carrier with PRB allocated, and measure the power spectrum density on the component carrier without PRB allocated.

[TP2: V2X UE output power within (0 -30) dBm]

Repeat the above steps 1~4 with the exception that making sure V2X UE transmission power to be -25.5dBm+/-4.5dB  for carrier frequency  f > 5GHz in step4.

Test procedure for PSBCH

[TP1: V2X UE output power > 0dBm]

1.	Set the V2X sidelink communication parameters for both the V2X sidelink capable UE (hereinafter referred to as V2X UE) and SS according to SL-V2X-Preconfiguration in TS 36.508[7] subclause 4.10.1 with the exception specified in clause 6.5.2.3G.3.4.3;

2.	V2X UE schedules the SLSS/PSBCH according to SL-V2X-Preconfiguration which is in line with the test configuration in Table 6.5.2.3G.3.4.1-2;

3.	Measure the V2X UE output power to make sure V2X UE transmission power to be 4.3dBm+/-4.3dB for carrier frequency  f > 5GHz;

4.	Measure In-band emission using Global In-Channel Tx-Test (Annex E)

[TP2: V2X UE output power within (0 -30) dBm]

Repeat the above steps 1~4 with the exception that making sure V2X UE transmission power to be -25.5dBm+/-4.5 dB for carrier frequency f > 5GHz in step3.

##### 6.5.2.3G.3.4.3	Message Contents

Message contents are according to TS 36.508 [7] subclause 4.10 Table Table 4.10.1.1-2 with the exceptions defined in clause 6.5.2.3G.1.4.3.:

##### 6.5.2.3G.3.5	Test Requirement

Each of the 20 In-band emissions results, derived in Annex E.4.3 shall not exceed the corresponding values in Table 6.5.2.3G.3.5-1.

Table 6.5.2.3G.3.5-1: Test requirements for in-band emissions (allocated component carrier)

| Parameter Description | Unit | Limit (Note 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | +0.8 |  | Any non-allocated (Note 2) |
| IQ Image | dB | -24.2 |  | Image frequencies (Note 3) |
| Carrier leakage | dBc | -24.2 | Output power f ≤ 3.0GHz: 3.2dBm ±3.2dB 3.0GHz < f ≤ 4.2GHz: 3.5 dBm ±3.5dB 4.2GHz < f ≤ 6GHz: 4.3 dBm ±4.3dB | LO frequency (Note 4) |
|  |  | -19.2 | Output power f ≤ 3.0GHz: -26.8 dBm ±3.2dB 3.0GHz < f ≤ 4.2GHz: -26.5 dBm ±3.5dB 4.2GHz < f ≤ 6GHz: -25.5dBm±4.5dB |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB - 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in NOTE 9. The limit is evaluated in each non-allocated RB. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs NOTE 3: Exceptions to the general limit are allowed for up to +1 RBs within a contiguous width of +1 non-allocated RBs. The measurement bandwidth is 1 RB. NOTE 4: Exceptions to the general limit are allowed for up to two contiguous non-allocated RBs. The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in the non-allocated RB to the measured total power in all allocated RBs. NOTE 5: is the Transmission Bandwidth (see Figure 5.4.2-1) not exceeding NOTE 6: is the Transmission Bandwidth Configuration (see Figure 5.4.2-1) of the component carrier with RBs allocated. NOTE 7: is the limit specified in Table 6.5.2.1G.3-1 for the modulation format used in the allocated RBs. NOTE 8: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. or for the first adjacent RB outside of the allocated bandwidth). NOTE 9: is the transmitted power per 180 kHz in allocated RBs, measured in dBm. NOTE 10: For V2X intra-band contiguous multi-carrier operation the in-band emission requirement (General, IQ Image or Carrier leakage) of each channel is calculated separately as Pgeneral, PSSCH,c(i), PIQ, PSSCH,c(i), PLO, PSSCH,c(i), Pgeneral, PSCCH,c(i), PIQ, PSCCH,c(i), PLO, PSCCH,c(i) for each carrier respectively. In-band emissions combined limit for the aggregated bandwidth is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the PsumIBE.CA, where PsumIBE.CA is the linear sum of Pgeneral, PSSCH,c(i), PIQ, PSSCH,c(i), PLO, PSSCH,c(i), Pgeneral, PSCCH,c(i), PIQ, PSCCH,c(i), PLO, PSCCH,c(i) for all carriers. |  |  |  |  |

The in-band emissions results, measured with the spectral test shall not exceed the corresponding values in Table 6.5.2.3G.3.5-2

Table 6.5.2.3G.3.5-2: Test requirements for in-band emissions (not allocated component carrier)

| Para-meter | Unit | Meas BW Note 1 | Limit |  | remark | Applicable Frequencies |
| --- | --- | --- | --- | --- | --- | --- |
| General | dB | BW of 1 RB (180KHz rectangular) | +0.8 |  | The reference value is the average power per allocated RB in the allocated component carrier | Any RB in the non allocated component carrier. The frequency raster of the RBs is derived when this component carrier is allocated with RBs |
| IQ Image | dB | BW of 1 RB (180KHz rectangular) | -24.2 Note 2 |  | The reference value is the average power per allocated RB in the allocated component carrier | The frequencies of thecontiguous non-allocated RBs are unknown. The frequency raster of the RBs is derived when this component carrier is allocated with RBs |
| Carrier leakage | dBc | BW of 1 RB (180KHz rectangular) | Note 3 |  | The reference value is the total power of the allocated RBs in the allocated component carrier | The frequencies of the up to 2 non-allocated RBs are unknown. The frequency raster of the RBs is derived when this component carrier is allocated with RBs |
|  |  |  | -24.2 | Output power > 0 dBm |  |  |
|  |  |  | -19.2 | -30 dBm ≤ Output power ≤ 0 dBm |  |  |
| Note1: Resolution BWs smaller than the measurement BW may be integrated to achieve the measurement bandwidth. Note 2: Exceptions to the general limit are allowed for up to +1 RBs within a contiguous width of +1 non-allocated RBs. Note 3: Exceptions to the general limit are allowed for up to two contiguous non-allocated RBs. Note 4: Note s1, 6, 7, 8, 9, 10 from Table 6.5.2A.3.0-1 apply for Table 6.5.2A.3.0-2 as well. Note 5: for measured non-allocated RB in the non allocated component carrier may take non-integer values when the carrier spacing between the CCs is not a multiple of RB. NOTE 6: For V2X intra-band contiguous multi-carrier operation the in-band emission requirement (General, IQ Image or Carrier leakage) of each channel is calculated separately as Pgeneral, PSSCH,c(i), PIQ, PSSCH,c(i), PLO, PSSCH,c(i), Pgeneral, PSCCH,c(i), PIQ, PSCCH,c(i), PLO, PSCCH,c(i) for each carrier respectively. In-band emissions combined limit for the aggregated bandwidth is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the PsumIBE.CA, where PsumIBE.CA is the linear sum of Pgeneral, PSSCH,c(i), PIQ, PSSCH,c(i), PLO, PSSCH,c(i), Pgeneral, PSCCH,c(i), PIQ, PSCCH,c(i), PLO, PSCCH,c(i) for all carriers. |  |  |  |  |  |  |

#### 6.5.2.4	EVM equalizer spectrum flatness

##### 6.5.2.4.1	Test Purpose

The zero-forcing equalizer correction applied in the EVM measurement process (as described in Annex E) must meet a spectrum flatness requirement for the EVM measurement to be valid. The EVM equalizer spectrum flatness is defined in terms of the maximum peak-to-peak ripple of the equalizer coefficients (dB) across the allocated uplink block variation in dB of the equalizer coefficients generated by the EVM measurement process. The EVM equalizer spectrum flatness requirement does not limit the correction applied to the signal in the EVM measurement process but for the EVM result to be valid, the equalizer correction that was applied must meet the EVM equalizer spectrum flatness minimum requirements. The basic measurement interval is the same as for EVM.

##### 6.5.2.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 8 and forward.

##### 6.5.2.4.3	Minimum conformance requirements

The peak-to-peak variation of the EVM equalizer coefficients contained within the frequency range of the uplink allocation shall not exceed the maximum ripple. The EVM equalizer spectrum flatness shall not exceed the values specified in Table 6.5.2.4.3-1 for normal conditions. For uplink allocations contained within both Range 1 and Range 2, the coefficients evaluated within each of these frequency ranges shall meet the corresponding ripple requirement and the following additional requirement: the relative difference between the maximum coefficient in Range 1 and the minimum coefficient in Range 2 must not be larger than 5 dB, and the relative difference between the maximum coefficient in Range 2 and the minimum coefficient in Range 1 must not be larger than 7 dB (see Figure 6.5.2.4.3-1).

The EVM equalizer spectrum flatness shall not exceed the values specified in Table 6.5.2.4.3-2 for extreme conditions. For uplink allocations contained within both Range 1 and Range 2, the coefficients evaluated within each of these frequency ranges shall meet the corresponding ripple requirement and the following additional requirement: the relative difference between the maximum coefficient in Range 1 and the minimum coefficient in Range 2 must not be larger than 6 dB, and the relative difference between the maximum coefficient in Range 2 and the minimum coefficient in Range 1 must not be larger than 10 dB (see Figure 6.5.2.4.3-1).

Table 6.5.2.4.3-1: Minimum requirements for EVM equalizer spectrum flatness (normal conditions)

| Frequency Range | Maximum Ripple [dB] |
| --- | --- |
| FUL_Meas – FUL_Low ≥ 3 MHz and FUL_High – FUL_Meas ≥ 3 MHz (Range 1) | 4 (p-p) |
| FUL_Meas – FUL_Low < 3 MHz or FUL_High – FUL_Meas < 3 MHz (Range 2) | 8 (p-p) |
| Note 1: FUL_Meas refers to the sub-carrier frequency for which the equalizer coefficient is evaluated Note 2: FUL_Low and FUL_High refer to each E-UTRA frequency band specified in Table 5.2-1 |  |

Table 6.5.2.4.3-2: Minimum requirements for EVM equalizer spectrum flatness (extreme conditions)

| FUL_Meas – FUL_Low ≥ 5 MHz and FUL_High – FUL_Meas ≥ 5 MHz (Range 1) | 4 (p-p) |
| --- | --- |
| FUL_Meas – FUL_Low < 5 MHz or FUL_High – FUL_Meas < 5 MHz (Range 2) | 12 (p-p) |
| Note 1: FUL_Meas refers to the sub-carrier frequency for which the equalizer coefficient is evaluated Note 2: FUL_Low and FUL_High refer to each E-UTRA frequency band specified in Table 5.2-1 |  |

Figure 6.5.2.4.3-1: The limits for EVM equalizer spectrum flatness with the maximum allowed variation of the coefficients indicated (the ETC minimum requirement within brackets)

The normative reference for this requirement is TS 36.101 clause 6.5.2.4.1.

##### 6.5.2.4.4	Test description

6.5.2.4.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.4.2.4.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.2.4.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  | See Table 6.5.1.4.1-1 |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  | See Table 6.5.1.4.1-1 |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  | See Table 6.5.1.4.1-1 |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
|  | Downlink Configuration | Uplink Configuration |  |  |
| Ch BW | N/A for EVM equalizer spectrum flatness testing | Mod'n | RB allocation |  |
|  |  |  | FDD | TDD |
| 1.4MHz |  | QPSK | 6 | 6 |
| 3MHz |  | QPSK | 15 | 15 |
| 5MHz |  | QPSK | 25 | 25 |
| 10MHz |  | QPSK | 50 | 50 |
| 15MHz |  | QPSK | 75 | 75 |
| 20MHz |  | QPSK | 100 | 100 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in Table 5.4.2.1-1. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Figure A.3.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to in Table 6.5.2.4.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.5.2.4.4.3.

6.5.2.4.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.5.2.4.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure spectrum flatness using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test.

6.5.2.4.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6.

##### 6.5.2.4.5	Test requirement

Each of the 20 spectrum flatness functions, shall derive four ripple results in Annex E.4.4, The derived results shall not exceed the values in Figure 6.5.2.4.5-1:

For normal conditions, the maximum ripple in Range 1 and Range 2 shall not exceed the values specified in Table 6.5.2.4.5-1 and the following additional requirement: the relative difference between the maximum coefficient in Range 1 and the minimum coefficient in Range 2 must not be larger than 6.4 dB, and the relative difference between the maximum coefficient in Range 2 and the minimum coefficient in Range 1 must not be larger than 8.4 dB (see Figure 6.5.2.4.5-1).

For extreme conditions, the maximum ripple in Range 1 and Range 2 shall not exceed the values specified in Table 6.5.2.4.5-2 and the following additional requirement: the relative difference between the maximum coefficient in Range 1 and the minimum coefficient in Range 2 must not be larger than 7.4 dB, and the relative difference between the maximum coefficient in Range 2 and the minimum coefficient in Range 1 must not be larger than 11.4 dB (see Figure 6.5.2.4.5-1).

Table 6.5.2.4.5-1: 	Test requirements for EVM equalizer spectrum flatness (normal conditions)

| Frequency Range | Maximum Ripple [dB] |
| --- | --- |
| FUL_Meas – FUL_Low ≥ 3 MHz and FUL_High – FUL_Meas ≥ 3 MHz (Range 1) | 5.4 (p-p) |
| FUL_Meas – FUL_Low < 3 MHz or FUL_High – FUL_Meas < 3 MHz (Range 2) | 9.4 (p-p) |
| Note 1: FUL_Meas refers to the sub-carrier frequency for which the equalizer coefficient is evaluated Note 2: FUL_Low and FUL_High refer to each E-UTRA frequency band specified in Table 5.2-1 |  |

Table 6.5.2.4.5-2: Test requirements for spectrum flatness (extreme conditions)

| Frequency Range | Maximum Ripple [dB] |
| --- | --- |
| FUL_Meas – FUL_Low ≥ 5 MHz and FUL_High – FUL_Meas ≥ 5 MHz (Range 1) | 5.4 (p-p) |
| FUL_Meas – FUL_Low < 5 MHz or FUL_High – FUL_Meas < 5 MHz (Range 2) | 13.4 (p-p) |
| Note 1: FUL_Meas refers to the sub-carrier frequency for which the equalizer coefficient is evaluated Note 2: FUL_Low and FUL_High refer to each E-UTRA frequency band specified in Table 5.2-1 |  |

Figure 6.5.2.4.5-1: The limits for EVM equalizer spectrum flatness with the maximum allowed variation of the coefficients indicated (the ETC minimum requirement within brackets)

#### 6.5.2.4E	EVM equalizer spectrum flatness for UE category 0

##### 6.5.2.4E.1	Test Purpose

Same test purpose as in clause 6.5.2.4.1.

##### 6.5.2.4E.2	Test applicability

This test case applies to all types of E-UTRA UE release 12 and forward of UE category 0.

##### 6.5.2.4E.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.2.4.3.

##### 6.5.2.4E.4	Test description

6.5.2.4E.4.1	Initial conditions

Same initial conditions as in clause 6.5.2.4.4.1 with following exceptions:

-	Instead of Table 6.5.2.4.4.1-1  use Table 6.5.2.4E.4.1-1.

-	Connect SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.3 using only main UE Tx/Rx antenna.

Table 6.5.2.4E.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  | See Table 6.5.1.4.1-1 |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  | See Table 6.5.1.4.1-1 |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  | See Table 6.5.1.4.1-1 |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
|  | Downlink Configuration | Uplink Configuration |  |  |
| Ch BW | N/A for EVM equalizer spectrum flatness testing | Mod'n | RB allocation |  |
|  |  |  | FDD and HD-FDD | TDD |
| 1.4MHz |  | QPSK | 6 | 6 |
| 3MHz |  | QPSK | 15 | 15 |
| 5MHz |  | QPSK | 25 | 25 |
| 10MHz |  | QPSK | 36 | 36 |
| 15MHz |  | QPSK | 36 | 36 |
| 20MHz |  | QPSK | 36 | 36 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in Table 5.4.2.1-1. |  |  |  |  |

6.5.2.4E.4.2	Test procedure

Same test procedure as in clause 6.5.2.4.4.2 with following exception for HD-FDD:

-	In step 3, slots with transient periods are not under test. Half-duplex guard sub frame are not under test.

6.5.2.4E.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6.

##### 6.5.2.4E.5	Test requirement

Each of the 20 spectrum flatness functions, shall derive four ripple results in Annex E.4.4, The derived results shall not exceed the values in Figure 6.5.2.4E.5-1:

For normal conditions, the maximum ripple in Range 1 and Range 2 shall not exceed the values specified in Table 6.5.2.4E.5-1 and the following additional requirement: the relative difference between the maximum coefficient in Range 1 and the minimum coefficient in Range 2 must not be larger than 6.4 dB, and the relative difference between the maximum coefficient in Range 2 and the minimum coefficient in Range 1 must not be larger than 8.4 dB (see Figure 6.5.2.4E.5-1).

For extreme conditions, the maximum ripple in Range 1 and Range 2 shall not exceed the values specified in Table 6.5.2.4E.5-2 and the following additional requirement: the relative difference between the maximum coefficient in Range 1 and the minimum coefficient in Range 2 must not be larger than 7.4 dB, and the relative difference between the maximum coefficient in Range 2 and the minimum coefficient in Range 1 must not be larger than 11.4 dB (see Figure 6.5.2.4E.5-1).

Table 6.5.2.4E.5-1: 	Test requirements for EVM equalizer spectrum flatness (normal conditions)

| Frequency Range | Maximum Ripple [dB] |
| --- | --- |
| FUL_Meas – FUL_Low ≥ 3 MHz and FUL_High – FUL_Meas ≥ 3 MHz (Range 1) | 5.4 (p-p) |
| FUL_Meas – FUL_Low < 3 MHz or FUL_High – FUL_Meas < 3 MHz (Range 2) | 9.4 (p-p) |
| Note 1: FUL_Meas refers to the sub-carrier frequency for which the equalizer coefficient is evaluated Note 2: FUL_Low and FUL_High refer to each E-UTRA frequency band specified in Table 5.2-1 |  |

Table 6.5.2.4E.5-2: Test requirements for spectrum flatness (extreme conditions)

| Frequency Range | Maximum Ripple [dB] |
| --- | --- |
| FUL_Meas – FUL_Low ≥ 5 MHz and FUL_High – FUL_Meas ≥ 5 MHz (Range 1) | 5.4 (p-p) |
| FUL_Meas – FUL_Low < 5 MHz or FUL_High – FUL_Meas < 5 MHz (Range 2) | 13.4 (p-p) |
| Note 1: FUL_Meas refers to the sub-carrier frequency for which the equalizer coefficient is evaluated Note 2: FUL_Low and FUL_High refer to each E-UTRA frequency band specified in Table 5.2-1 |  |

Figure 6.5.2.4E.5-1: The limits for EVM equalizer spectrum flatness with the maximum allowed variation of the coefficients indicated (the ETC minimum requirement within brackets)

#### 6.5.2.4EA	EVM equalizer spectrum flatness for UE category M1

##### 6.5.2.4EA.1	Test Purpose

Same test purpose as in clause 6.5.2.4.1.

##### 6.5.2.4EA.2	Test applicability

This test case applies to all types of E-UTRA UE release 13 and forward of UE category M1.

##### 6.5.2.4EA.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.2.4.3.

##### 6.5.2.4EA.4	Test description

6.5.2.4EA.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in sub-clause 5.2E. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.4.2.4EA.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in A.2. Configurations of PDSCH and MPDCCH before measurement are specified in Annex C.2

Table 6.5.2.4EA.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  | See Table 6.5.1.4.1-1 |  |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  | See Table 6.5.1.4.1-1 |  |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  | 5MHz |  |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  | Downlink Configuration | Uplink Configuration |  |  |  |
| Ch BW | N/A for EVM equalizer spectrum flatness testing | Mod'n | RB allocation |  |  |
|  |  |  | FDD and HD-FDD | TDD | Narrowband index1 |
| 5MHz |  | QPSK | 6 | 6 | 0 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4 |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to in Table 6.5.2.4EA.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [7] clause 5.2A.2AA. Message contents are defined in clause 6.5.2.4EA.4.3.

6.5.2.4EA.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via MPDCCH DCI format 6-0A for C_RNTI to schedule the UL RMC according to Table 6.5.2.4EA.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure spectrum flatness using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test. For HD-FDD slots with transient periods are not under test. Half-duplex guard sub frame are not under test.

6.5.2.4EA.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the condition CEModeA.

##### 6.5.2.4EA.5	Test requirement

Each of the 20 spectrum flatness functions, shall derive four ripple results in Annex E.4.4, The derived results shall not exceed the values in Figure 6.5.2.4EA.5-1:

For normal conditions, the maximum ripple in Range 1 and Range 2 shall not exceed the values specified in Table 6.5.2.4EA.5-1 and the following additional requirement: the relative difference between the maximum coefficient in Range 1 and the minimum coefficient in Range 2 must not be larger than 6.4 dB, and the relative difference between the maximum coefficient in Range 2 and the minimum coefficient in Range 1 must not be larger than 8.4 dB (see Figure 6.5.2.4EA.5-1).

For extreme conditions, the maximum ripple in Range 1 and Range 2 shall not exceed the values specified in Table 6.5.2.4EA.5-2 and the following additional requirement: the relative difference between the maximum coefficient in Range 1 and the minimum coefficient in Range 2 must not be larger than 7.4 dB, and the relative difference between the maximum coefficient in Range 2 and the minimum coefficient in Range 1 must not be larger than 11.4 dB (see Figure 6.5.2.4EA.5-1).

Table 6.5.2.4EA.5-1: Test requirements for EVM equalizer spectrum flatness (normal conditions)

| Frequency Range | Maximum Ripple [dB] |
| --- | --- |
| FUL_Meas – FUL_Low ≥ 3 MHz and FUL_High – FUL_Meas ≥ 3 MHz (Range 1) | 5.4 (p-p) |
| FUL_Meas – FUL_Low < 3 MHz or FUL_High – FUL_Meas < 3 MHz (Range 2) | 9.4 (p-p) |
| Note 1: FUL_Meas refers to the sub-carrier frequency for which the equalizer coefficient is evaluated Note 2: FUL_Low and FUL_High refer to each E-UTRA frequency band specified in Table 5.2-1 |  |

Table 6.5.2.4EA.5-2: Test requirements for spectrum flatness (extreme conditions)

| Frequency Range | Maximum Ripple [dB] |
| --- | --- |
| FUL_Meas – FUL_Low ≥ 5 MHz and FUL_High – FUL_Meas ≥ 5 MHz (Range 1) | 5.4 (p-p) |
| FUL_Meas – FUL_Low < 5 MHz or FUL_High – FUL_Meas < 5 MHz (Range 2) | 13.4 (p-p) |
| Note 1: FUL_Meas refers to the sub-carrier frequency for which the equalizer coefficient is evaluated Note 2: FUL_Low and FUL_High refer to each E-UTRA frequency band specified in Table 5.2-1 |  |

Figure 6.5.2.4EA.5-1: The limits for EVM equalizer spectrum flatness with the maximum allowed variation of the coefficients indicated (the ETC minimum requirement within brackets)

#### 6.5.2.4EB	EVM equalizer spectrum flatness for UE category 1bis

##### 6.5.2.4EB.1	Test purpose

Same test purpose as in clause 6.5.2.4.

##### 6.5.2.4EB.2	Test applicability

This test applies to all types of E-UTRA UE release 13 and forward of UE category 1bis.

##### 6.5.2.4EB.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.2.4.3.

##### 6.5.2.4EB.4	Test description

6.5.2.4EB.4.1	Initial conditions

Same initial conditions as in clause 6.5.2.4.4.1 with the following exception:

-	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.3 using only main UE Tx/Rx antenna.

6.5.2.4EB.4.2	Test procedure

Same test procedure as in clause 6.5.2.4.4.2.

6.5.2.4EB.4.3	Message contents

Same message contents as in clause 6.5.2.4.4.3.

##### 6.5.2.4EB.5	Test requirement

Each of the 20 spectrum flatness functions, shall derive four ripple results in Annex E.4.4, The derived results shall not exceed the values in Figure 6.5.2.4EB.5-1:

For normal conditions, the maximum ripple in Range 1 and Range 2 shall not exceed the values specified in Table 6.5.2.4EB.5-1 and the following additional requirement: the relative difference between the maximum coefficient in Range 1 and the minimum coefficient in Range 2 must not be larger than 6.4 dB, and the relative difference between the maximum coefficient in Range 2 and the minimum coefficient in Range 1 must not be larger than 8.4 dB (see Figure 6.5.2.4EB.5-1).

For extreme conditions, the maximum ripple in Range 1 and Range 2 shall not exceed the values specified in Table 6.5.2.4EB.5-2 and the following additional requirement: the relative difference between the maximum coefficient in Range 1 and the minimum coefficient in Range 2 must not be larger than 7.4 dB, and the relative difference between the maximum coefficient in Range 2 and the minimum coefficient in Range 1 must not be larger than 11.4 dB (see Figure 6.5.2.4EB.5-1).

Table 6.5.2.4EB.5-1: 	Test requirements for EVM equalizer spectrum flatness (normal conditions)

| Frequency Range | Maximum Ripple [dB] |
| --- | --- |
| FUL_Meas – FUL_Low ≥ 3 MHz and FUL_High – FUL_Meas ≥ 3 MHz (Range 1) | 5.4 (p-p) |
| FUL_Meas – FUL_Low < 3 MHz or FUL_High – FUL_Meas < 3 MHz (Range 2) | 9.4 (p-p) |
| Note 1: FUL_Meas refers to the sub-carrier frequency for which the equalizer coefficient is evaluated Note 2: FUL_Low and FUL_High refer to each E-UTRA frequency band specified in Table 5.2-1 |  |

Table 6.5.2.4EB.5-2: Test requirements for spectrum flatness (extreme conditions)

| Frequency Range | Maximum Ripple [dB] |
| --- | --- |
| FUL_Meas – FUL_Low ≥ 5 MHz and FUL_High – FUL_Meas ≥ 5 MHz (Range 1) | 5.4 (p-p) |
| FUL_Meas – FUL_Low < 5 MHz or FUL_High – FUL_Meas < 5 MHz (Range 2) | 13.4 (p-p) |
| Note 1: FUL_Meas refers to the sub-carrier frequency for which the equalizer coefficient is evaluated Note 2: FUL_Low and FUL_High refer to each E-UTRA frequency band specified in Table 5.2-1 |  |

Figure 6.5.2.4EB.5-1: The limits for EVM equalizer spectrum flatness with the maximum allowed variation of the coefficients indicated (the ETC minimum requirement within brackets)

#### 6.5.2.4EC	EVM equalizer spectrum flatness for UE category M2

##### 6.5.2.4EC.1	Test Purpose

Same test purpose as in clause 6.5.2.4.1.

##### 6.5.2.4EC.2	Test applicability

This test case applies to all types of E-UTRA UE release 14 and forward of UE category M2.

##### 6.5.2.4EC.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.2.4.3.

##### 6.5.2.4EC.4	Test description

6.5.2.4EC.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in sub-clause 5.2E. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.5.2.4EC.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in A.2. Configurations of PDSCH and MPDCCH before measurement are specified in Annex C.2

Table 6.5.2.4EC.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  | Low range, Mid range, High range |  |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  | 5MHz, Highest |  |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  | Downlink Configuration | Uplink Configuration |  |  |  |
| Ch BW |  | Mod'n | RB allocation |  |  |
|  |  |  | FDD and HD-FDD | TDD | Narrowband index1 |
| 5MHz | N/A for EVM equalizer spectrum | QPSK | 24 | 24 | 0 |
| 10MHz | flatness testing | QPSK | 24 | 24 | 0 |
| 15MHz |  | QPSK | 24 | 24 | 0 |
| 20MHz |  | QPSK | 24 | 24 | 0 |
| Note 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.4. |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to in Table 6.5.2.4EC.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [7] clause 5.2A.2AA. Message contents are defined in clause 6.5.2.4EC.4.3.

6.5.2.4EC.4.2	Test procedure

Same test procedure as in clause 6.5.2.4EA.4.2 with the following exception.

-	Instead of Table 6.5.2.4EA.4.1-1  use Table 6.5.2.4EC.4.1-1.

6.5.2.4EC.4.3	Message contents

Same message contents as in clause 6.5.2.4EA.4.3.

##### 6.5.2.4EC.5	Test requirement

Same test requirement as in clause 6.5.2.4EA.5.

#### 6.5.2.4G	EVM equalizer spectrum flatness for V2X transmission

##### 6.5.2.4G.1	EVM equalizer spectrum flatness for V2X Communication / Non-concurrent with E-UTRA uplink transmissions

###### 6.5.2.4G.1.1	Test Purpose

The zero-forcing equalizer correction applied in the EVM measurement process (as described in Annex E) must meet a spectrum flatness requirement for the EVM measurement to be valid. The EVM equalizer spectrum flatness is defined in terms of the maximum peak-to-peak ripple of the equalizer coefficients (dB) across the V2X sidelink allocated block variation in dB of the equalizer coefficients generated by the EVM measurement process. The EVM equalizer spectrum flatness requirement does not limit the correction applied to the signal in the EVM measurement process but for the EVM result to be valid, the equalizer correction that was applied must meet the EVM equalizer spectrum flatness minimum requirements. The basic measurement interval is the same as for EVM.

###### 6.5.2.4G.1.2	Test applicability

This test case applies to all types of UE that supports V2X Sidelink communication and Band 47.

###### 6.5.2.4G.1.3	Minimum conformance requirements

The peak-to-peak variation of the EVM equalizer coefficients contained within the frequency range of the V2X sidelink allocation shall not exceed the maximum ripple. The EVM equalizer spectrum flatness shall not exceed the values specified in Table 6.5.2.4G.1.3-1 for normal conditions. For V2X sidelink allocations contained within both Range 1 and Range 2, the coefficients evaluated within each of these frequency ranges shall meet the corresponding ripple requirement and the following additional requirement: the relative difference between the maximum coefficient in Range 1 and the minimum coefficient in Range 2 must not be larger than 5 dB, and the relative difference between the maximum coefficient in Range 2 and the minimum coefficient in Range 1 must not be larger than 7 dB (see Figure 6.5.2.4G.1.3-1).

The EVM equalizer spectrum flatness shall not exceed the values specified in Table 6.5.2.4G.1.3-2 for extreme conditions. For uplink allocations contained within both Range 1 and Range 2, the coefficients evaluated within each of these frequency ranges shall meet the corresponding ripple requirement and the following additional requirement: the relative difference between the maximum coefficient in Range 1 and the minimum coefficient in Range 2 must not be larger than 6 dB, and the relative difference between the maximum coefficient in Range 2 and the minimum coefficient in Range 1 must not be larger than 10 dB (see Figure 6.5.2.4G.1.3-1).

For intra-band contiguous multi-carrier operation the above EVM equalizer spectrum flatness requirement shall apply for each component carrier.

Table 6.5.2.4G.1.3-1: Minimum requirements for EVM equalizer spectrum flatness (normal conditions)

| Frequency Range | Maximum Ripple [dB] |
| --- | --- |
| FUL_Meas – FUL_Low ≥ 3 MHz and FUL_High – FUL_Meas ≥ 3 MHz (Range 1) | 4 (p-p) |
| FUL_Meas – FUL_Low < 3 MHz or FUL_High – FUL_Meas < 3 MHz (Range 2) | 8 (p-p) |
| Note 1: FUL_Meas refers to the sub-carrier frequency for which the equalizer coefficient is evaluated Note 2: FUL_Low and FUL_High refer to each E-UTRA frequency band specified in Table 5.2G-1 |  |

Table 6.5.2.4G.1.3-2: Minimum requirements for EVM equalizer spectrum flatness (extreme conditions)

| FUL_Meas – FUL_Low ≥ 5 MHz and FUL_High – FUL_Meas ≥ 5 MHz (Range 1) | 4 (p-p) |
| --- | --- |
| FUL_Meas – FUL_Low < 5 MHz or FUL_High – FUL_Meas < 5 MHz (Range 2) | 12 (p-p) |
| Note 1: FUL_Meas refers to the sub-carrier frequency for which the equalizer coefficient is evaluated Note 2: FUL_Low and FUL_High refer to each E-UTRA frequency band specified in Table 5.2G-1 |  |

Figure 6.5.2.4G.1.3-1: The limits for EVM equalizer spectrum flatness with the maximum allowed variation of the coefficients indicated (the ETC minimum requirement within brackets)

The normative reference for this requirement is TS 36.101 clause 6.5.2.4.1.

###### 6.5.2.4G.1.4	Test description

6.5.2.4G.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in sub-clause 5.2G. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.5.2.4G.1.4.1-1 the details of the sidelink reference measurement channels (RMCs) are specified in A.8.3 and the GNSS configuration in TS 36.508 [7] subclause 4.11.

Table 6.5.2.4G.1.4.1-1: Test Configuration Table for PSSCH and PSCCH

| Initial Conditions |  |  |
| --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  | NC |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  | Low range, Mid range, High range |
| Test Channel Bandwidths (as specified in subclause 5.4.2G) |  | Lowest, Highest |
| Test Parameters for Channel Bandwidths |  |  |
|  | V2X Configuration to Transmit |  |
| Ch BW | Mod'n | PSSCH RB Allocation |
| 10MHz | QPSK | 48@2 |
| 20MHz | QPSK | 96@2 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in specified in subclause 5.4.2G. |  |  |

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 36.508[7] Figure A.92.

2.	The parameter settings for the V2X sidelink transmission over PC5 for both SS and UE are pre-configured according to TS 36.508 [7] subclause 4.10.1 with the exception specified in clause 6.5.2.4G.1.4.3-1.

3.	The V2X reference measurement channel is set according to Table 6.5.2.4G.1.4.1-1

4.	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS36.508 [7] Table 4.11.2-2. Geographical area #1 is also pre-configured in the UE.

5.	Propagation conditions are set according to Annex B.0;

6.	Ensure the UE is in State 5A-V2X in Transmit Mode according to TS 36.508 [7] clause 4.5.9.

7.	The GNSS simulator is triggered to start step 1 of Scenario #1 to simulate a location in the centre of Geographical area #1. Wait for the UE to acquire the GNSS signal and start to transmit.

6.5.2.4G.1.4.2	Test procedure

1.	The V2X UE schedules the V2X RMC with transmission power at PUMAX level according to SL-V2X-Preconfiguration which is in line with the test configuration in Table 6.5.2.4G.1.4.1-1;

2.	Measure spectrum flatness using Global In-Channel Tx-Test (Annex E).

6.5.2.4G.1.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause Table 6.8.2.1-1with the following exceptions:

Table 6.5.2.4G.1.4.3-1: SL-V2X-Preconfiguration-r14

| Derivation Path: 36.508 Table 4.10.1.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-V2X-Preconfiguration-r14 ::= SEQUENCE { |  |  |  |
| v2x-PreconfigFreqList-r14 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | One entry of v2x-PreconfigFreqInfo |  |  |
| SL-V2X-PreconfigFreqInfo-r14[1] ::= SEQUENCE { |  |  |  |
| v2x-CommPreconfigGeneral-r14 SEQUENCE { |  | SL-PreconfigGeneral-r12 |  |
| maxTxPower-r12 | 23 |  | Power class3 |
|  | 26 |  | Power class2 |
| sl-bandwidth-r12 | n50 | 10Mhz |  |
|  | n100 | 20Mhz |  |
| } |  |  |  |
| v2x-CommPreconfigSync-r14 SEQUENCE { |  | SL-PreconfigSync-r12 |  |
| syncOffsetIndicators-r14 SEQUENCE { |  |  |  |
| syncOffsetIndicator1-r14 | Not Present | Double synchronization signal transmission |  |
| } |  |  |  |
| } |  |  |  |
| v2x-CommRxPoolList-r14 SEQUENCE (SIZE (1..maxSL-V2X-RxPoolPreconf-r14)) OF SL-V2X-PreconfigCommPool-r14 { | 1 entry |  |  |
| SL-V2X-PreconfigCommPool-r14[1] | SL-V2X-PreconfigCommPool-r14-DEFAULT using condition BITMAP_6 and exception listed in Table 6.5.2.4G.1.4.3-2 |  |  |
| } |  |  |  |
| v2x-CommTxPoolList-r14 SEQUENCE (SIZE (1..maxSL-V2X-TxPoolPreconf-r14)) OF SL-V2X-PreconfigCommPool-r14 { |  |  |  |
| SL-V2X-PreconfigCommPool-r14[1] | SL-V2X-PreconfigCommPool-r14-DEFAULT using condition BITMAP_6 and exception listed in Table 6.5.2.4G.1.4.3-2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.2.4G.1.4.3-2: SL-V2X-PreconfigCommPool-r14-DEFAULT

| Derivation Path: 36.508 Table 4.6.3-20J |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-CommResroucePoolV2X-r14-DEFAULT ::= SEQUENCE { |  |  |  |
| sl-Subframe-r14 CHOICE { |  |  |  |
| bs20-r14 | 11111111111111111111 |  | BITMAP_6 |
| } |  |  |  |
| dataTxParameters-r14 SEQUENCE { |  |  |  |
| p0-r12 | 31 | It’s set to the maximum value to disable its effect on transmission power. |  |
| } |  |  |  |
| } |  |  |  |

###### 6.5.2.4G.1.5	Test requirement

Each of the 20 spectrum flatness functions, shall derive four ripple results in Annex E.4.4, The derived results shall not exceed the values in Figure 6.5.2.4G.5-1:

For normal conditions, the maximum ripple in Range 1 and Range 2 shall not exceed the values specified in Table 6.5.2.4G.5-1and the following additional requirement: the relative difference between the maximum coefficient in Range 1 and the minimum coefficient in Range 2 must not be larger than 6.4 dB, and the relative difference between the maximum coefficient in Range 2 and the minimum coefficient in Range 1 must not be larger than 8.4 dB (see Figure 6.5.2.4G.5-1).

For extreme conditions, the maximum ripple in Range 1 and Range 2 shall not exceed the values specified in Table 6.5.2.4G.5-2 and the following additional requirement: the relative difference between the maximum coefficient in Range 1 and the minimum coefficient in Range 2 must not be larger than 7.4dB, and the relative difference between the maximum coefficient in Range 2 and the minimum coefficient in Range 1 must not be larger than 11.4 dB (see Figure 6.5.2.4G.1.5-1).

Table 6.5.2.4G.5-1: Test requirements for EVM equalizer spectrum flatness (normal conditions)

| Frequency Range | Maximum Ripple [dB] |
| --- | --- |
| FUL_Meas – FUL_Low ≥ 3 MHz and FUL_High – FUL_Meas ≥ 3 MHz (Range 1) | 5.4 (p-p) |
| FUL_Meas – FUL_Low < 3 MHz or FUL_High – FUL_Meas < 3 MHz (Range 2) | 9.4 (p-p) |
| Note 1: FUL_Meas refers to the sub-carrier frequency for which the equalizer coefficient is evaluated Note 2: FUL_Low and FUL_High refer to each E-UTRA frequency band specified in Table 5.2-1 |  |

Table 6.5.2.4G.5-2: Test requirements for spectrum flatness (extreme conditions)

| Frequency Range | Maximum Ripple [dB] |
| --- | --- |
| FUL_Meas – FUL_Low ≥ 5 MHz and FUL_High – FUL_Meas ≥ 5 MHz (Range 1) | 5.4 (p-p) |
| FUL_Meas – FUL_Low < 5 MHz or FUL_High – FUL_Meas < 5 MHz (Range 2) | 13.4 (p-p) |
| Note 1: FUL_Meas refers to the sub-carrier frequency for which the equalizer coefficient is evaluated Note 2: FUL_Low and FUL_High refer to each E-UTRA frequency band specified in Table 5.2-1 |  |

Figure 6.5.2.4G.5-1: The limits for EVM equalizer spectrum flatness with the maximum allowed variation of the coefficients indicated (the ETC minimum requirement within brackets)

##### 6.5.2.4G.2	Void

##### 6.5.2.4G.3	EVM equalizer spectrum flatness for V2X Communication / Intra-band contiguous multi-carrier operation

6.5.2.4G.3.1	Test purpose

The zero-forcing equalizer correction applied in the EVM measurement process (as described in Annex E) must meet a spectrum flatness requirement for the EVM measurement to be valid. The EVM equalizer spectrum flatness is defined in terms of the maximum peak-to-peak ripple of the equalizer coefficients (dB) across the V2X sidelink allocated block variation in dB of the equalizer coefficients generated by the EVM measurement process. The EVM equalizer spectrum flatness requirement does not limit the correction applied to the signal in the EVM measurement process but for the EVM result to be valid, the equalizer correction that was applied must meet the EVM equalizer spectrum flatness minimum requirements. The basic measurement interval is the same as for EVM.

###### 6.5.2.4G.3.2	Test Applicability

This test case applies to all types of UE that supports Intra-band contiguous multi-carrier operation V2X Sidelink communication and Band 47.

###### 6.5.2.4G.3.3	Minimum conformance requirements

The requirements of subclause 6.5.2.4.3 shall apply for V2X transmissions.

For intra-band contiguous multi-carrier operation the EVM equalizer spectrum flatness requirement of subclause 6.5.2.4.3 shall apply for each component carrier.

The normative reference for this requirement is TS 36.101 [2] subclause 6.5.2G.3 and subclause 6.5.2G.4

###### 6.5.2.4G.3.4	Test Description

6.5.2.4G.3.4.1	Initial Conditions

Initial conditions specify the set of UE parameters that UE needs be test in and the steps that SS should take with UE to reach the correct measurement state.

The initial test conditions consist of environment conditions, test frequencies and channel bandwidth based on E-UTRAN operating bands specified in subclauses 5.2G and 5.4.2G and propagation conditions. All these configurations shall be tested with applicable parameters for each channel bandwidth, and are shown in table 6.5.2.4G.3.4.1-1; the details of reference channel are specified in sub-clause A.8.3 and the reference GNSS configuration in TS 36.508 [7] subclause 4.11.

6.5.2.4G.3.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |
| Test CC Combination setting(NRB_agg) as specified in subclause 5.4.2G |  |  | Lowest NRB_agg, Highest NRB_agg |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| V2X Multi-carrier Configuration |  | V2X PSSCH Allocation |  |  |
| CC1 NRB | CC2 NRB | CC1 & CC2 Mod |  | CC1 and CC2 RB Allocation |
| 50 | 50 | QPSK |  | 48@2+48@2 |

1. Connect the SS and GNSS simulator to UE antenna connectors according to the figure A.89ain TS 36.508 [7] Annex A.

2. The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS36.508 [7] Table 4.11.2-2. Geographical area #1 is also pre-configured in the UE.

3. Propagation conditions are set according to Annex B.0.

4. Trigger the UE to reset UTC time. (NOTE: The UTC time reset may be performed by MMI or AT command (+CUTCR).)

5. The GNSS simulator is triggered to start step 1 of Scenario #1 to simulate a location in the centre of Geographical area #1. Wait for the UE to acquire the GNSS signal and start to transmit.

6. Ensure the UE is in the Side link State 5A-V2X in Transmit Mode, according to TS 36.508 [7] clause4.5.9. Message contents are defined in clause 6.5.2.4G.3.4.3

6.5.2.4G.3.4.2	Test Procedure

1.	The V2X UE schedules the V2X RMC with transmission power at PUMAX level according to SL-V2X-Preconfiguration which reflects the test configuration in Table 6.5.2.4G.3.4.1-1;

2.	Measure spectrum flatness for each component carrier using Global In-Channel Tx-Test (Annex E).

6.5.2.4G.3.4.3	Message Contents

Message contents are according to TS 36.508 [7] subclause 4.10 Table 4.10.1.1-2 with the exceptions defined in clause 6.5.2.4G.1.4.3.

###### 6.5.2.4G.3.5	Test requirement

Each of the 20 spectrum flatness functions, shall derive four ripple results in Annex E.9.1, The derived results for each component carrier shall not exceed the values in Figure 6.5.2.4G.3.5-1:

For normal conditions, the maximum ripple in Range 1 and Range 2 shall not exceed the values specified in Table 6.5.2.4G.3.5-1and the following additional requirement: the relative difference between the maximum coefficient in Range 1 and the minimum coefficient in Range 2 must not be larger than 6.4 dB, and the relative difference between the maximum coefficient in Range 2 and the minimum coefficient in Range 1 must not be larger than 8.4 dB (see Figure 6.5.2.4G.3.5-1).

For extreme conditions, the maximum ripple in Range 1 and Range 2 shall not exceed the values specified in Table 6.5.2.4G.3.5-2 and the following additional requirement: the relative difference between the maximum coefficient in Range 1 and the minimum coefficient in Range 2 must not be larger than 7.4dB, and the relative difference between the maximum coefficient in Range 2 and the minimum coefficient in Range 1 must not be larger than 11.4 dB (see Figure 6.5.2.4G.3.5-1).

Table 6.5.2.4G.3.5-1: Test requirements for EVM equalizer spectrum flatness (normal conditions)

| Frequency Range | Maximum Ripple [dB] |
| --- | --- |
| FUL_Meas – FUL_Low ≥ 3 MHz and FUL_High – FUL_Meas ≥ 3 MHz (Range 1) | 5.4 (p-p) |
| FUL_Meas – FUL_Low < 3 MHz or FUL_High – FUL_Meas < 3 MHz (Range 2) | 9.4 (p-p) |
| Note 1: FUL_Meas refers to the sub-carrier frequency for which the equalizer coefficient is evaluated Note 2: FUL_Low and FUL_High refer to each E-UTRA frequency band specified in Table 5.2-1 |  |

Table 6.5.2.4G.3.5-2: Test requirements for spectrum flatness (extreme conditions)

| Frequency Range | Maximum Ripple [dB] |
| --- | --- |
| FUL_Meas – FUL_Low ≥ 5 MHz and FUL_High – FUL_Meas ≥ 5 MHz (Range 1) | 5.4 (p-p) |
| FUL_Meas – FUL_Low < 5 MHz or FUL_High – FUL_Meas < 5 MHz (Range 2) | 13.4 (p-p) |
| Note 1: FUL_Meas refers to the sub-carrier frequency for which the equalizer coefficient is evaluated Note 2: FUL_Low and FUL_High refer to each E-UTRA frequency band specified in Table 5.2-1 |  |

Figure 6.5.2.4G.3.5-1: The limits for EVM equalizer spectrum flatness with the maximum allowed variation of the coefficients indicated (the ETC minimum requirement within brackets)

### 6.5.2A	Transmit modulation for CA

The requirements in this clause apply with PCC and SCC in the UL configured and activated: PCC with PRB allocation and SCC without PRB allocation and without CSI reporting and SRS configured.

#### 6.5.2A.1	Error Vector Magnitude (EVM) for CA

##### 6.5.2A.1.0	Minimum conformance requirements

For inter-band carrier aggregation, EVM measurements are evaluated for each component carrier, and for the different modulations schemes, the EVM requirements shall not exceed the values specified in Table 6.5.2.1.3-1, if CA is configured in uplink.

For the intra-band non-contiguous carrier aggregation, EVM measurements are evaluated for each component carrier, and for the different modulations schemes, the EVM requirements shall not exceed the values specified in Table 6.5.2A.1.0-1, if CA is configured in uplink.

For the intra-band contiguous carrier aggregation, EVM measurements are evaluated for each component carrier, and for the different modulations schemes, the EVM requirements shall not exceed the values specified in Table 6.5.2A.1.0-1, if CA is configured in uplink.

When a single component carrier is configured Table 6.5.2.1.3-1 apply.

The EVM requirements are according to Table 6.5.2A.1.0-1 if CA is configured in uplink with the parameters defined in Table 6.5.2.1.3-2 and 6.5.2.1_2.3-2 for 256QAM.

Table 6.5.2A.1.0-1: Minimum requirements for Error Vector Magnitude

| Parameter | Unit | Average EVM Level per CC | Reference Signal EVM Level |
| --- | --- | --- | --- |
| QPSK or BPSK | % | 17.5 | 17.5 |
| 16QAM | % | 12.5 | 12.5 |
| 64 QAM | % | 8 | 8 |
| 256 QAM | % | 3.5 | 3.5 |

The normative reference for this requirement is TS 36.101 [2] clause 6.5.2A.1.

##### 6.5.2A.1.1	Error Vector Magnitude (EVM) for CA (intra-band contiguous DL CA and UL CA)

6.5.2A.1.1.1	Test Purpose

For the intra-band contiguous carrier aggregation, the Error Vector Magnitude requirement should be defined for each component carrier. Requirement applies for the allocated component carrier, when all other component carriers are activated, but not allocated.

Similar transmitter impairment removal procedures are applied for CA waveform before EVM calculation as is specified for non-CA waveform in sub-section 6.5.2.1.

6.5.2A.1.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 10 and forward that support intra-band contiguous DL CA and UL CA.

6.5.2A.1.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5.2A.1.0.

6.5.2A.1.1.4	Test description

6.5.2A.1.1.4.1		Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on E-UTRA CA configurations specified in table 5.4.2A.1-1. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 6.5.2A.1.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.2A.1.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. PCC and SCC mapped onto component carriers CC according to the notation: PCC-SCC: CCi-CCj, which means PCC on CCi and SCC on CCj, with CCi/j frequencies defined in the corresponding intra-band contiguous CA band in TS36.508 [7] as above. |  |  |  |  | See Table 6.5.1A.1.4.1-1: PCC-SCC: CC1-CC2 and CC2-CC1 |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  | See Table 6.5.1A.1.4.1-1: |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |
|  | PCC NRB Note 1 | SCCs NRB Note 1 | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 50 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 2 | 50 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 3 | 50 | 50 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 4 | 50 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 5 | 50 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 6 | 50 | 50 |  | 16QAM | 12 | P_12@38 | S_0@0 | - | - |
| 7 | 50 | 25 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 8 | 50 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 9 | 50 | 25 |  | QPSK | 25 | P_25@25 | S_0@0 | - | - |
| 10 | 50 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 11 | 50 | 25 |  | QPSK | 8 | P_8@42 | S_0@0 | - | - |
| 12 | 50 | 25 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 13 | 50 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 14 | 50 | 25 |  | 16QAM | 25 | P_25@25 | S_0@0 | - | - |
| 15 | 50 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 16 | 50 | 25 |  | 16QAM | 8 | P_8@42 | S_0@0 | - | - |
| 17 | 25 | 50 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 18 | 25 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 19 | 25 | 50 |  | QPSK | 12 | P_12@13 | S_0@0 | - | - |
| 20 | 25 | 50 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 21 | 25 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 22 | 25 | 50 |  | 16QAM | 12 | P_12@13 | S_0@0 | - | - |
| 23 | 75 | 75 | N.A. | QPSK | 75 | P_75@0 | S_0@0 | - | - |
| 24 | 75 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 25 | 75 | 75 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| 26 | 75 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 | - | - |
| 27 | 75 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 | - | - |
| 28 | 75 | 75 |  | 16QAM | 16 | P_16@59 | S_0@0 | - | - |
| 29 | 100 | 25 |  | QPSK | 100 | P_100@0 | S_0@0 | - | - |
| 30 | 100 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 31 | 100 | 25 |  | QPSK | 25 | P_25@75 | S_0@0 | - | - |
| 320 | 100 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 33 | 100 | 25 |  | QPSK | 8 | P_8@92 | S_0@0 | - | - |
| 34 | 100 | 25 |  | 16QAM | 100 | P_100@0 | S_0@0 | - | - |
| 35 | 100 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 36 | 100 | 25 |  | 16QAM | 25 | P_25@75 | S_0@0 | - | - |
| 37 | 100 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 38 | 100 | 25 |  | 16QAM | 8 | P_8@92 | S_0@0 | - | - |
| 39 | 25 | 100 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 40 | 25 | 100 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 41 | 25 | 100 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 42 | 25 | 100 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 43 | 25 | 100 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 44 | 25 | 100 |  | 16QAM | 8 | P_8@17 | S_0@0 | - | - |
| 453 | 100 | 50 |  | QPSK | 100 | P_100@0 | S_0@0 | - | - |
| 46 | 100 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 47 | 100 | 50 |  | QPSK | 50 | P_50@50 | S_0@0 | - | - |
| 48 | 100 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 49 | 100 | 50 |  | QPSK | 12 | P_12@88 | S_0@0 | - | - |
| 50 | 100 | 50 |  | 16QAM | 100 | P_100@0 | S_0@0 | - | - |
| 51 | 100 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 52 | 100 | 50 |  | 16QAM | 50 | P_50@50 | S_0@0 | - | - |
| 53 | 100 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 54 | 100 | 50 |  | 16QAM | 12 | P_12@88 | S_0@0 | - | - |
| 55 | 50 | 100 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 56 | 50 | 100 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 57 | 50 | 100 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 58 | 50 | 100 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 59 | 50 | 100 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 60 | 50 | 100 |  | 16QAM | 12 | P_12@38 | S_0@0 | - | - |
| 61 | 100 | 75 |  | QPSK | 100 | P_100@0 | S_0@0 | - | - |
| 62 | 100 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 | - | - |
| 63 | 100 | 75 |  | QPSK | 75 | P_75@25 | S_0@0 | - | - |
| 64 | 100 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 65 | 100 | 75 |  | QPSK | 16 | P_16@84 | S_0@0 | - | - |
| 66 | 100 | 75 |  | 16QAM | 100 | P_100@0 | S_0@0 | - | - |
| 67 | 100 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 | - | - |
| 68 | 100 | 75 |  | 16QAM | 75 | P_75@25 | S_0@0 | - | - |
| 69 | 100 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 | - | - |
| 70 | 100 | 75 |  | 16QAM | 16 | P_16@84 | S_0@0 | - | - |
| 71 | 75 | 100 |  | QPSK | 75 | P_75@0 | S_0@0 | - | - |
| 72 | 75 | 100 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 73 | 75 | 100 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| 74 | 75 | 100 |  | 16QAM | 75 | P_75@0 | S_0@0 | - | - |
| 75 | 75 | 100 |  | 16QAM | 16 | P_16@0 | S_0@0 | - | - |
| 76 | 75 | 100 |  | 16QAM | 16 | P_16@59 | S_0@0 | - | - |
| 77 | 100 | 100 |  | QPSK | 100 | P_100@0 | S_0@0 | - | - |
| 78 | 100 | 100 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| 79 | 100 | 100 |  | QPSK | 18 | P_18@82 | S_0@0 | - | - |
| 80 | 100 | 100 |  | 16QAM | 100 | P_100@0 | S_0@0 | - | - |
| 81 | 100 | 100 |  | 16QAM | 18 | P_18@0 | S_0@0 | - | - |
| 82 | 100 | 100 |  | 16QAM | 18 | P_18@82 | S_0@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A, Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.5.2A.1.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause [5.2A.2]. Message contents are defined in clause 6.5.2A.1.1.4.3.

6.5.2A.1.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.5.2A.3.2.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.5.2A.1.1.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

6.	Measure the EVM and  on PCC using Global In-Channel Tx-Test (Annex E).

7.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is –36.8dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -36.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

8.	Measure the EVM and  on PCC using Global In-Channel Tx-Test (Annex E).

6.5.2A.1.1.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

6.5.2A.1.1.5	Test requirement

The PUSCH EVM derived in E.4.2 shall not exceed 17,5 % for QPSK and BPSK, 12,5% for 16 QAM.

The PUSCH derived in E.4.6.2 shall not exceed 17,5 % when embedded with data symbols of QPSK and BPSK, 12,5% for 16 QAM.

##### 6.5.2A.1.1_1	EVM for CA (intra-band contiguous DL CA and UL CA) with UL 64QAM

6.5.2A.1.1_1.1	Test Purpose

Same test purpose as in clause 6.5.2A.1.1.1

6.5.2A.1.1_1.2	Test applicability

This test applies to E-UTRA UE from release 13 and forward that support intra-band contiguous DL CA and UL CA and UL 64QAM.

6.5.2A.1.1_1.3	Minimum conformance requirements

The EVM requirement for 64QAM is according to Table 6.5.2A.1.1_1.3-1 if CA is configured in uplink.

Table 6.5.2A.1.1_1.3-1: Minimum requirements for Error Vector Magnitude

| Parameter | Unit | Average EVM Level per CC | Reference Signal EVM Level |
| --- | --- | --- | --- |
| 64QAM | % | 8 | 8 |

6.5.2A.1.1_1.4	Test description

6.5.2A.1.1_1.4.1		Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on E-UTRA CA configurations specified in table 5.4.2A.1-1. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 6.5.2A.1.1_1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.2A.1.1_1.4.1-1: Test Configuration Table

| Initial Conditions |  |
| --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 | NC |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. PCC and SCC mapped onto component carriers CC according to the notation: PCC-SCC: CCi-CCj, which means PCC on CCi and SCC on CCj, with CCi/j frequencies defined in the corresponding intra-band contiguous CA band in TS36.508 [7] as above. | See Table 6.5.1A.1.4.1-1: PCC-SCC: CC1-CC2 and CC2-CC1 |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration | See Table 6.5.1A.1.4.1-1: |
| Test Parameters for CA Configurations |  |
| Use the content of Table 6.5.2A.1.1.4.1-1 and change the modulation of all QPSK test points to 64QAM. Config IDs with 16QAM modulation shall not be tested. |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A, Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.5.2A.1.1_1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause [5.2A.2]. Message contents are defined in clause 6.5.2A.1.1_1.4.3.

6.5.2A.1.1_1.4.2	Test procedure

Same test procedure as in clause 6.5.2A.1.1.4.2

6.5.2A.1.1_1.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. For SCC configuration there are no additional message contents.

6.5.2A.1.1_1.5	Test requirement

The PUSCH EVM derived in E.4.2 shall not exceed 8% for 64QAM.

The PUSCH derived in E.4.6.2 shall not exceed [8%] for 64QAM.

##### 6.5.2A.1.1_2	EVM for CA (intra-band contiguous DL CA and UL CA) with UL 256QAM

6.5.2A.1.1_2.1	Test Purpose

Same test purpose as in clause 6.5.2A.1.1.1

6.5.2A.1.1_2.2	Test applicability

This test case applies to all types of E-UTRA UE release 14 and forward that support intra-band contiguous DL CA and UL CA and UL 256QAM.

6.5.2A.1.1_2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5.2A.1.0

6.5.2A.1.1_2.4	Test description

6.5.2A.1.1_2.4.1		Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on E-UTRA CA configurations specified in table 5.4.2A.1-1. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 6.5.2A.1.1_2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.2A.1.1_2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. PCC and SCC mapped onto component carriers CC according to the notation: PCC-SCC: CCi-CCj, which means PCC on CCi and SCC on CCj, with CCi/j frequencies defined in the corresponding intra-band contiguous CA band in TS36.508 [7] as above. |  |  |  |  | See Table 6.5.1A.1.4.1-1: PCC-SCC: CC1-CC2 and CC2-CC1 |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  | See Table 6.5.1A.1.4.1-1: |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |
|  | PCC NRB Note 1 | SCCs NRB Note 1 | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 50 | 50 | N/A for this test | 256QAM | 50 | P_50@0 | S_0@0 | - | - |
| 2 | 50 | 25 |  | 256QAM | 50 | P_50@0 | S_0@0 | - | - |
| 3 | 25 | 50 |  | 256QAM | 25 | P_25@0 | S_0@0 | - | - |
| 4 | 75 | 50 |  | 256QAM | 75 | P_75@0 | S_0@0 | - | - |
| 5 | 75 | 75 |  | 256QAM | 75 | P_75@0 | S_0@0 | - | - |
| 6 | 100 | 25 |  | 256QAM | 100 | P_100@0 | S_0@0 | - | - |
| 7 | 25 | 100 |  | 256QAM | 25 | P_25@0 | S_0@0 | - | - |
| 8 | 75 | 50 |  | 256QAM | 75 | P_75@0 | S_0@0 | - | - |
| 9 | 100 | 50 |  | 256QAM | 100 | P_100@0 | S_0@0 | - | - |
| 10 | 50 | 100 |  | 256QAM | 50 | P_50@0 | S_0@0 | - | - |
| 11 | 100 | 75 |  | 256QAM | 100 | P_100@0 | S_0@0 | - | - |
| 12 | 75 | 100 |  | 256QAM | 75 | P_75@0 | S_0@0 | - | - |
| 13 | 100 | 100 |  | 256QAM | 100 | P_100@0 | S_0@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg, only the first of those is tested, according to the order on the Test Configuration Table list. |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A, Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.5.2A.1.1_2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.5.2A.1.1_2.4.3.

6.5.2A.1.1_2.4.2	Test procedure

Same as in clause 6.5.2A.1.1.4.2 with the following exceptions:

-	Instead of Table 6.5.2A.1.1.4.1-1  use Table 6.5.2A.1.1_2.4.1-1.

-	Instead of UE output power -36.8dBm ±3.2dB  -26.8dBm ±3.2dB for carrier frequency f ≤ 3.0GHz.

-	Instead of UE output power -36.5dBm ±3.5dB  -26.5dBm ±3.5dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

6.5.2A.1.1_2.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. For SCC configuration there are no additional message contents.

6.5.2A.1.1_2.5	Test requirement

The PUSCH EVM derived in Annex E.4.2 shall not exceed the values in Table 6.5.2.1_2.5-1 for 256QAM.

The PUSCH derived in Annex E.4.6.2 shall not exceed the values in Table 6.5.2.1_2.5-1 when embedded with data symbols of 256QAM.

##### 6.5.2A.1.2	Error Vector Magnitude (EVM) for CA (inter-band DL CA and UL CA)

6.5.2A.1.2.1	Test Purpose

For the inter-band carrier aggregation, the Error Vector Magnitude requirement should be defined for each component carrier. Requirement applies for the allocated component carrier, when all other component carriers are activated, but not allocated.

Similar transmitter impairment removal procedures are applied for CA waveform before EVM calculation as is specified for non-CA waveform in sub-section 6.5.2.1.

6.5.2A.1.2.2	Test applicability

This test case applies to all types of E-UTRA UE Release 11 and forward that support inter-band DL CA and UL CA.

This test case also applies to all types of E-UTRA UE release 14 and forward that supports uplink LAA.

6.5.2A.1.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5.2A.1.0.

6.5.2A.1.2.4	Test description

6.5.2A.1.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on E-UTRA CA configurations specified in table 5.4.2A.1-2. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 6.5.2A.1.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.2A.1.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  | Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |
|  | PCC NRB Note 1 | SCCs NRB Note 1 | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 6 | 25 | N.A. | QPSK | 1 | P_1@0 | S_0@0 | - | - |
| 2 | 6 | 25 |  | QPSK | 1 | P_1@5 | S_0@0 | - | - |
| 3 | 6 | 25 |  | QPSK | 6 | P_6@0 | S_0@0 | - | - |
| 4 | 6 | 25 |  | 16QAM | 1 | P_1@0 | S_0@0 | - | - |
| 5 | 6 | 25 |  | 16QAM | 1 | P_1@5 | S_0@0 | - | - |
| 6 | 6 | 25 |  | 16QAM | 6 | P_6@0 | S_0@0 | - | - |
| 7 | 6 | 50 |  | QPSK | 1 | P_1@0 | S_0@0 | - | - |
| 8 | 6 | 50 |  | QPSK | 1 | P_1@5 | S_0@0 | - | - |
| 9 | 6 | 50 |  | QPSK | 6 | P_6@0 | S_0@0 | - | - |
| 10 | 6 | 50 |  | 16QAM | 1 | P_1@0 | S_0@0 | - | - |
| 11 | 6 | 50 |  | 16QAM | 1 | P_1@5 | S_0@0 | - | - |
| 12 | 6 | 50 |  | 16QAM | 6 | P_6@0 | S_0@0 | - | - |
| 13 | 15 | 25 |  | QPSK | 4 | P_4@0 | S_0@0 | - | - |
| 14 | 15 | 25 |  | QPSK | 4 | P_4@11 | S_0@0 | - | - |
| 15 | 15 | 25 |  | QPSK | 15 | P_15@0 | S_0@0 | - | - |
| 16 | 15 | 25 |  | 16QAM | 4 | P_4@0 | S_0@0 | - | - |
| 17 | 15 | 25 |  | 16QAM | 4 | P_4@11 | S_0@0 | - | - |
| 18 | 15 | 25 |  | 16QAM | 15 | P_15@0 | S_0@0 | - | - |
| 19 | 25 | 6 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 20 | 25 | 6 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 21 | 25 | 6 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 22 | 25 | 6 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 23 | 25 | 6 |  | 16QAM | 8 | P_8@17 | S_0@0 | - | - |
| 24 | 25 | 6 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 25 | 25 | 15 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 26 | 25 | 15 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 27 | 25 | 15 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 28 | 25 | 15 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 29 | 25 | 15 |  | 16QAM | 8 | P_8@17 | S_0@0 | - | - |
| 30 | 25 | 15 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 31 | 25 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 32 | 25 | 25 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 33 | 25 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 34 | 25 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 35 | 25 | 25 |  | 16QAM | 8 | P_8@17 | S_0@0 | - | - |
| 36 | 25 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 37 | 25 | 50 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 38 | 25 | 50 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 39 | 25 | 50 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 40 | 25 | 50 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 41 | 25 | 50 |  | 16QAM | 8 | P_8@17 | S_0@0 | - | - |
| 42 | 25 | 50 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 43 | 50 | 6 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 44 | 50 | 6 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 45 | 50 | 6 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 46 | 50 | 6 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 47 | 50 | 6 |  | 16QAM | 12 | P_12@38 | S_0@0 | - | - |
| 48 | 50 | 6 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 49 | 50 | 25 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 50 | 50 | 25 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 51 | 50 | 25 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 52 | 50 | 25 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 53 | 50 | 25 |  | 16QAM | 12 | P_12@38 | S_0@0 | - | - |
| 54 | 50 | 25 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 55 | 50 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 56 | 50 | 50 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 57 | 50 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 58 | 50 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 59 | 50 | 50 |  | 16QAM | 12 | P_12@38 | S_0@0 | - | - |
| 60 | 50 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 61 | 50 | 100 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 62 | 50 | 100 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 63 | 50 | 100 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 64 | 50 | 100 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 65 | 50 | 100 |  | 16QAM | 12 | P_12@38 | S_0@0 | - | - |
| 66 | 50 | 100 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 67 | 75 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 68 | 75 | 75 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| 69 | 75 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 | - | - |
| 70 | 75 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 | - | - |
| 71 | 75 | 75 |  | 16QAM | 16 | P_16@59 | S_0@0 | - | - |
| 72 | 75 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 | - | - |
| 73 | 75 | 100 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 74 | 75 | 100 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| 75 | 75 | 100 |  | QPSK | 75 | P_75@0 | S_0@0 | - | - |
| 76 | 75 | 100 |  | 16QAM | 16 | P_16@0 | S_0@0 | - | - |
| 77 | 75 | 100 |  | 16QAM | 16 | P_16@59 | S_0@0 | - | - |
| 78 | 75 | 100 |  | 16QAM | 75 | P_75@0 | S_0@0 | - | - |
| 79 | 100 | 50 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| 80 | 100 | 50 |  | QPSK | 18 | P_18@82 | S_0@0 | - | - |
| 81 | 100 | 50 |  | QPSK | 100 | P_100@0 | S_0@0 | - | - |
| 82 | 100 | 50 |  | 16QAM | 18 | P_18@0 | S_0@0 | - | - |
| 83 | 100 | 50 |  | 16QAM | 18 | P_18@82 | S_0@0 | - | - |
| 84 | 100 | 50 |  | 16QAM | 100 | P_100@0 | S_0@0 | - | - |
| 85 | 100 | 75 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| 86 | 100 | 75 |  | QPSK | 18 | P_18@82 | S_0@0 | - | - |
| 87 | 100 | 75 |  | QPSK | 100 | P_100@0 | S_0@0 | - | - |
| 88 | 100 | 75 |  | 16QAM | 18 | P_18@0 | S_0@0 | - | - |
| 89 | 100 | 75 |  | 16QAM | 18 | P_18@82 | S_0@0 | - | - |
| 90 | 100 | 75 |  | 16QAM | 100 | P_100@0 | S_0@0 | - | - |
| 91 | 100 | 100 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| 92 | 100 | 100 |  | QPSK | 18 | P_18@82 | S_0@0 | - | - |
| 93 | 100 | 100 |  | QPSK | 100 | P_100@0 | S_0@0 | - | - |
| 94 | 100 | 100 |  | 16QAM | 18 | P_18@0 | S_0@0 | - | - |
| 95 | 100 | 100 |  | 16QAM | 18 | P_18@82 | S_0@0 | - | - |
| 96 | 100 | 100 |  | 16QAM | 100 | P_100@0 | S_0@0 | - | - |
| 97 | 25 | 75 |  | QPSK | 8 | P_8@0 | S_0@0 |  |  |
| 98 | 25 | 75 |  | QPSK | 8 | P_8@17 | S_0@0 |  |  |
| 99 | 25 | 75 |  | QPSK | 25 | P_25@0 | S_0@0 |  |  |
| 100 | 25 | 75 |  | 16QAM | 8 | P_8@0 | S_0@0 |  |  |
| 101 | 25 | 75 |  | 16QfAM | 8 | P_8@17 | S_0@0 |  |  |
| 102 | 25 | 75 |  | 16QAM | 25 | P_25@0 | S_0@0 |  |  |
| 103 | 75 | 25 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 104 | 75 | 25 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| 105 | 75 | 25 |  | QPSK | 75 | P_75@0 | S_0@0 | - | - |
| 106 | 75 | 25 |  | 16QAM | 16 | P_16@0 | S_0@0 | - | - |
| 107 | 75 | 25 |  | 16QAM | 16 | P_16@59 | S_0@0 | - | - |
| 108 | 75 | 25 |  | 16QAM | 75 | P_75@0 | S_0@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: The frequencies of PCC and SCC shall be switched and tested in each configuration. |  |  |  |  |  |  |  |  |  |

Table 6.5.2A.1.2.4.1-2: Test Configuration Table for Uplink LAA CA

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  |  | Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |  |
| Test Parameters for the uplink LAA CA Configurations |  |  |  |  |  |  |  |  |  |  |  |
| Config ID |  | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |  |
|  |  | PCC NRB Note 1 | SCCs NRB Note 1 | PCC & SCC RB allocation |  | NRB_alloc | PCC RB Allocation (LCRB @ RBstart) SCC RB Allocations(L@ RBstart) Note2 |  |  |  |
| 1 | 25 | 100 | NA | QPSK | 10 | P_0@0 | S_1@0 |  |  |
| 2 | 25 | 100 |  | QPSK | 10 | P_0@0 | S_1@9 |  |  |
| 3 | 25 | 100 |  | QPSK | 100 | P_0@0 | S_10@0 |  |  |
| 4 | 25 | 100 |  | 16QAM | 10 | P_0@0 | S_1@0 |  |  |
| 5 | 25 | 100 |  | 16QAM | 10 | P_0@0 | S_1@9 |  |  |
| 6 | 25 | 100 |  | 16QAM | 100 | P_0@0 | S_10@0 |  |  |
| 7 | 50 | 100 |  | QPSK | 10 | P_0@0 | S_1@0 |  |  |
| 8 | 50 | 100 |  | QPSK | 10 | P_0@0 | S_1@9 |  |  |
| 9 | 50 | 100 |  | QPSK | 100 | P_0@0 | S_10@0 |  |  |
| 10 | 50 | 100 |  | 16QAM | 10 | P_0@0 | S_1@0 |  |  |
| 11 | 50 | 100 |  | 16QAM | 10 | P_0@0 | S_1@9 |  |  |
| 12 | 50 | 100 |  | 16QAM | 100 | P_0@0 | S_10@0 |  |  |
| 13 | 75 | 100 |  | QPSK | 10 | P_0@0 | S_1@0 |  |  |
| 14 | 75 | 100 |  | QPSK | 10 | P_0@0 | S_1@9 |  |  |
| 15 | 75 | 100 |  | QPSK | 100 | P_0@0 | S_10@0 |  |  |
| 16 | 75 | 100 |  | 16QAM | 10 | P_0@0 | S_1@0 |  |  |
| 17 | 75 | 100 |  | 16QAM | 10 | P_0@0 | S_1@9 |  |  |
| 18 | 75 | 100 |  | 16QAM | 100 | P_0@0 | S_10@0 |  |  |
| 19 | 100 | 100 |  | QPSK | 10 | P_0@0 | S_1@0 |  |  |
| 20 | 100 | 100 |  | QPSK | 10 | P_0@0 | S_1@9 |  |  |
| 21 | 100 | 100 |  | QPSK | 100 | P_0@0 | S_10@0 |  |  |
| 22 | 100 | 100 |  | 16QAM | 10 | P_0@0 | S_1@0 |  |  |
| 23 | 100 | 100 |  | 16QAM | 10 | P_0@0 | S_1@9 |  |  |
| 24 | 100 | 100 |  | 16QAM | 100 | P_0@0 | S_10@0 |  |  |
| Note 1: CA Configuration, Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths for uplink LAA are specified in Table 5.4.2A.1-2. Note 2: For eLAA SCC, the RBstart and L are defined in TS 36.213[10] clause 8.1.4. |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL RMC is set according to Table 6.5.2A.1.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.5.2A.1.2.4.3.

6.5.2A.1.2.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.5.2A.3.2.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 (or DCI format 0A for SCC in the case of uplink LAA) for C_RNTI to schedule the UL RMC according to Table 6.5.2A.1.2.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

6.	Measure the EVM and  on PCC using Global In-Channel Tx-Test (Annex E).

7.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is –36.8dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -36.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

8.	Measure the EVM and  on PCC using Global In-Channel Tx-Test (Annex E).

6.5.2A.1.2.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

For SCC in the case of uplink LAA, message contents are according to TS 36.508[7] subclause4.6 with the following exceptions specified in Tables 6.5.2A.1.2.4.3-1 and 6.5.2A.1.2.4.3-2.

Table 6.5.2A.1.2.4.3-1 LogicalChannelConfig-DRB (Initial Condition, step 14, Table 4.5.2.3-1)

| Derivation Path: 36.508 Table 4.8.2.1.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LogicalChannelConfig-DRB ::= SEQUENCE { |  |  |  |  |
| ul-SpecificParameters SEQUENCE { |  |  |  |  |
| laa-UL-Allowed-r14 |  | true |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 6.5.2A.1.2.4.3-2 PhysicalConfigDedicatedSCell (Step 2, test procedure)

| Derivation Path: 36.508 Table 4.6.3-6A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicatedSCell-r10-DEFAULT ::= SEQUENCE { |  |  |  |
| laa-SCellConfiguration-v1430 ::= SEQUENCE { |  |  |  |
|  | NULL |  |  |
| } |  |  |  |
| } |  |  |  |

6.5.2A.1.2.5	Test requirement

The PUSCH EVM derived in Annex E.4.2 shall not exceed 17,5 % for QPSK and BPSK, 12,5% for 16 QAM. The PUSCH derived in Annex E.4.6.2 shall not exceed 17,5 % when embedded with data symbols of QPSK and BPSK, 12,5% for 16 QAM.

##### 6.5.2A.1.2_1	Error Vector Magnitude (EVM) for CA (inter-band DL CA and UL CA) for UL 64QAM

6.5.2A.1.2_1.1	Test Purpose

For the inter-band carrier aggregation, the Error Vector Magnitude requirement should be defined for each component carrier. Requirement applies for the allocated component carrier, when all other component carriers are activated, but not allocated.

Similar transmitter impairment removal procedures are applied for CA waveform before EVM calculation as is specified for non-CA waveform in sub-section 6.5.2.1.

6.5.2A.1.2_1.2	Test applicability

This test case applies to all types of E-UTRA UE Release 13 and forward that support inter-band DL CA and UL CA and UL 64QAM.

This test case also applies to all types of E-UTRA UE release 14 and forward that support uplink LAA.

6.5.2A.1.2_1.3	Minimum conformance requirements

The EVM requirement for 64QAM is according to Table 6.5.2A.1.1_1.3-1 if CA is configured in uplink.

Table 6.5.2A.1.2_1.3-1: Minimum requirements for Error Vector Magnitude

| Parameter | Unit | Average EVM Level | Reference Signal EVM Level |
| --- | --- | --- | --- |
| 64QAM | % | 8 | 8 |

6.5.2A.1.2_1.4	Test description

6.5.2A.1.2_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on E-UTRA CA configurations specified in table 5.4.2A.1-2. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 6.5.2A.1.2_1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.2A.1.2_1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  | Mid range for PCC and SCC |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  | Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |
|  | PCC NRB Note 1 | SCCs NRB Note 1 | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 6 | 25 | N/A for this test | 64QAM | 1 | P_1@0 | S_0@0 | - | - |
| 2 | 6 | 25 |  | 64QAM | 1 | P_1@5 | S_0@0 | - | - |
| 3 | 6 | 25 |  | 64QAM | 6 | P_6@0 | S_0@0 | - | - |
| 4 | 6 | 50 |  | 64QAM | 1 | P_1@0 | S_0@0 | - | - |
| 5 | 6 | 50 |  | 64QAM | 1 | P_1@5 | S_0@0 | - | - |
| 6 | 6 | 50 |  | 64QAM | 6 | P_6@0 | S_0@0 | - | - |
| 7 | 15 | 25 |  | 64QAM | 4 | P_4@0 | S_0@0 | - | - |
| 8 | 15 | 25 |  | 64QAM | 4 | P_4@11 | S_0@0 | - | - |
| 9 | 15 | 25 |  | 64QAM | 15 | P_15@0 | S_0@0 | - | - |
| 10 | 25 | 6 |  | 64QAM | 8 | P_8@0 | S_0@0 | - | - |
| 11 | 25 | 6 |  | 64QAM | 8 | P_8@17 | S_0@0 | - | - |
| 12 | 25 | 6 |  | 64QAM | 25 | P_25@0 | S_0@0 | - | - |
| 13 | 25 | 15 |  | 64QAM | 8 | P_8@0 | S_0@0 | - | - |
| 14 | 25 | 15 |  | 64QAM | 8 | P_8@17 | S_0@0 | - | - |
| 15 | 25 | 15 |  | 64QAM | 25 | P_25@0 | S_0@0 | - | - |
| 16 | 25 | 25 |  | 64QAM | 8 | P_8@0 | S_0@0 | - | - |
| 17 | 25 | 25 |  | 64QAM | 8 | P_8@17 | S_0@0 | - | - |
| 18 | 25 | 25 |  | 64QAM | 25 | P_25@0 | S_0@0 | - | - |
| 19 | 25 | 50 |  | 64QAM | 8 | P_8@0 | S_0@0 | - | - |
| 20 | 25 | 50 |  | 64QAM | 8 | P_8@17 | S_0@0 | - | - |
| 21 | 25 | 50 |  | 64QAM | 25 | P_25@0 | S_0@0 | - | - |
| 22 | 50 | 6 |  | 64QAM | 12 | P_12@0 | S_0@0 | - | - |
| 23 | 50 | 6 |  | 64QAM | 12 | P_12@38 | S_0@0 | - | - |
| 24 | 50 | 6 |  | 64QAM | 50 | P_50@0 | S_0@0 | - | - |
| 25 | 50 | 25 |  | 64QAM | 12 | P_12@0 | S_0@0 | - | - |
| 26 | 50 | 25 |  | 64QAM | 12 | P_12@38 | S_0@0 | - | - |
| 27 | 50 | 25 |  | 64QAM | 50 | P_50@0 | S_0@0 | - | - |
| 28 | 50 | 50 |  | 64QAM | 12 | P_12@0 | S_0@0 | - | - |
| 29 | 50 | 50 |  | 64QAM | 12 | P_12@38 | S_0@0 | - | - |
| 30 | 50 | 50 |  | 64QAM | 50 | P_50@0 | S_0@0 | - | - |
| 31 | 50 | 100 |  | 64QAM | 12 | P_12@0 | S_0@0 | - | - |
| 32 | 50 | 100 |  | 64QAM | 12 | P_12@38 | S_0@0 | - | - |
| 33 | 50 | 100 |  | 64QAM | 50 | P_50@0 | S_0@0 | - | - |
| 34 | 75 | 75 |  | 64QAM | 16 | P_16@0 | S_0@0 | - | - |
| 35 | 75 | 75 |  | 64QAM | 16 | P_16@59 | S_0@0 | - | - |
| 36 | 75 | 75 |  | 64QAM | 75 | P_75@0 | S_0@0 | - | - |
| 37 | 75 | 100 |  | 64QAM | 16 | P_16@0 | S_0@0 | - | - |
| 38 | 75 | 100 |  | 64QAM | 16 | P_16@59 | S_0@0 | - | - |
| 39 | 75 | 100 |  | 64QAM | 75 | P_75@0 | S_0@0 | - | - |
| 40 | 100 | 50 |  | 64QAM | 18 | P_18@0 | S_0@0 | - | - |
| 41 | 100 | 50 |  | 64QAM | 18 | P_18@82 | S_0@0 | - | - |
| 42 | 100 | 50 |  | 64QAM | 100 | P_100@0 | S_0@0 | - | - |
| 43 | 100 | 75 |  | 64QAM | 18 | P_18@0 | S_0@0 | - | - |
| 44 | 100 | 75 |  | 64QAM | 18 | P_18@82 | S_0@0 | - | - |
| 45 | 100 | 75 |  | 64QAM | 100 | P_100@0 | S_0@0 | - | - |
| 46 | 100 | 100 |  | 64QAM | 18 | P_18@0 | S_0@0 | - | - |
| 47 | 100 | 100 |  | 64QAM | 18 | P_18@82 | S_0@0 | - | - |
| 48 | 100 | 100 |  | 64QAM | 100 | P_100@0 | S_0@0 | - | - |
| 49 | 75 | 25 |  | 64QAM | 16 | P_16@0 | S_0@0 | - | - |
| 50 | 75 | 25 |  | 64QAM | 16 | P_16@59 | S_0@0 | - | - |
| 51 | 75 | 25 |  | 64QAM | 75 | P_75@0 | S_0@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: The frequencies of PCC and SCC shall be switched and tested in each configuration. |  |  |  |  |  |  |  |  |  |

Table 6.5.2A.1.2_1.4.1-2: Test Configuration Table for Uplink LAA CA

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  | Mid range for PCC and SCC |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  | Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |
|  | PCC NRB Note 1 | SCCs NRB Note 1 | PCC & SCC RB allocation |  | NRB_alloc | PCC RB Allocation (LCRB @ RBstart) SCC RB Allocations(L@ RBstart) Note2 |  |  |  |
| 1 | 25 | 100 |  | 64QAM | 10 | P_0@0 | S_1@0 |  |  |
| 2 | 25 | 100 |  | 64QAM | 10 | P_0@0 | S_1@9 |  |  |
| 3 | 25 | 100 |  | 64QAM | 100 | P_0@0 | S_10@0 |  |  |
| 4 | 50 | 100 |  | 64QAM | 10 | P_0@0 | S_1@0 |  |  |
| 5 | 50 | 100 |  | 64QAM | 10 | P_0@0 | S_1@9 |  |  |
| 6 | 50 | 100 |  | 64QAM | 100 | P_0@0 | S_10@0 |  |  |
| 7 | 75 | 100 |  | 64QAM | 10 | P_0@0 | S_1@0 |  |  |
| 8 | 75 | 100 |  | 64QAM | 10 | P_0@0 | S_1@9 |  |  |
| 9 | 75 | 100 |  | 64QAM | 100 | P_0@0 | S_10@0 |  |  |
| 10 | 100 | 100 |  | 64QAM | 10 | P_0@0 | S_1@0 |  |  |
| 11 | 100 | 100 |  | 64QAM | 10 | P_0@0 | S_1@9 |  |  |
| 12 | 100 | 100 |  | 64QAM | 100 | P_0@0 | S_10@0 |  |  |
| Note 1: CA Configuration, Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths for uplink LAA are specified in Table 5.4.2A.1-2. Note 2: For eLAA SCC, the RBstart and L are defined in TS 36.213[10] clause 8.1.4. |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL RMC is set according to Table 6.5.2A.1.2_1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.5.2A.1.2_1.4.3.

6.5.2A.1.2_1.4.2	Test procedure

Same test procedure as in clause 6.5.2A.1.2.4.2.

6.5.2A.1.2_1.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

For SCC in the case of uplink LAA, message contents are according to TS 36.508[7] subclause4.6 with the exceptions specified in Tables 6.5.2A.1.2.4.3-1 and 6.5.2A.1.2.4.3-2.

6.5.2A.1.2_1.5	Test requirement

The PUSCH EVM derived in Annex E.4.2 shall not exceed 8% for 64QAM.

The PUSCH derived in Annex E.4.6.2 shall not exceed 8% when embedded with data symbols of 64QAM.

##### 6.5.2A.1.2_2	Error Vector Magnitude (EVM) for CA (inter-band DL CA and UL CA) for UL 256QAM

6.5.2A.1.2_2.1	Test Purpose

For the inter-band carrier aggregation, the Error Vector Magnitude requirement should be defined for each component carrier. Requirement applies for the allocated component carrier, when all other component carriers are activated, but not allocated.

Similar transmitter impairment removal procedures are applied for CA waveform before EVM calculation as is specified for non-CA waveform in sub-section 6.5.2.1.

6.5.2A.1.2_2.2	Test applicability

This test case applies to all types of E-UTRA UE release 14 and forward that support inter-band DL CA and UL CA and UL 256QAM.

6.5.2A.1.2_2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5.2A.1.0

6.5.2A.1.2_2.4	Test description

6.5.2A.1.2_2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on E-UTRA CA configurations specified in table 5.4.2A.1-2. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 6.5.2A.1.2_2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.2A.1.2_2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  | Mid range for PCC and SCC |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  | Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |
|  | PCC NRB Note 1 | SCCs NRB Note 1 | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 6 | 25 | N/A for this test | 256QAM | 6 | P_6@0 | S_0@0 | - | - |
| 2 | 6 | 50 |  | 256QAM | 6 | P_6@0 | S_0@0 | - | - |
| 3 | 15 | 25 |  | 256QAM | 15 | P_15@0 | S_0@0 | - | - |
| 4 | 25 | 6 |  | 256QAM | 25 | P_25@0 | S_0@0 | - | - |
| 5 | 25 | 15 |  | 256QAM | 25 | P_25@0 | S_0@0 | - | - |
| 6 | 25 | 25 |  | 256QAM | 25 | P_25@0 | S_0@0 | - | - |
| 7 | 25 | 50 |  | 256QAM | 25 | P_25@0 | S_0@0 | - | - |
| 8 | 50 | 6 |  | 256QAM | 50 | P_50@0 | S_0@0 | - | - |
| 9 | 50 | 25 |  | 256QAM | 50 | P_50@0 | S_0@0 | - | - |
| 10 | 50 | 50 |  | 256QAM | 50 | P_50@0 | S_0@0 | - | - |
| 11 | 50 | 100 |  | 256QAM | 50 | P_50@0 | S_0@0 | - | - |
| 12 | 75 | 75 |  | 256QAM | 75 | P_75@0 | S_0@0 | - | - |
| 13 | 75 | 100 |  | 256QAM | 75 | P_75@0 | S_0@0 | - | - |
| 14 | 100 | 50 |  | 256QAM | 100 | P_100@0 | S_0@0 | - | - |
| 15 | 100 | 75 |  | 256QAM | 100 | P_100@0 | S_0@0 | - | - |
| 16 | 100 | 100 |  | 256QAM | 100 | P_100@0 | S_0@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: The frequencies of PCC and SCC shall be switched and tested in each configuration. |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL RMC is set according to Table 6.5.2A.1.2_2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.5.2A.1.2_2.4.3.

6.5.2A.1.2_2.4.2	Test procedure

Same as in clause 6.5.2A.1.2.4.2 with the following exceptions:

-	Instead of Table 6.5.2A.1.2.4.1-1  use Table 6.5.2A.1.2_2.4.1-1.

-	Instead of UE output power -36.8dBm ±3.2dB  -26.8dBm ±3.2dB for carrier frequency f ≤ 3.0GHz.

-	Instead of UE output power -36.5dBm ±3.5dB  -26.5dBm ±3.5dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

6.5.2A.1.2_2.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

6.5.2A.1.2_2.5	Test requirement

The PUSCH EVM derived in Annex E.4.2 shall not exceed the values in Table 6.5.2.1_2.5-1 for 256QAM.

The PUSCH derived in Annex E.4.6.2 shall not exceed the values in Table 6.5.2.1_2.5-1 when embedded with data symbols of 256QAM.

##### 6.5.2A.1.3	Error Vector Magnitude (EVM) for CA (intra-band non-contiguous DL CA and UL CA)

6.5.2A.1.3.1	Test Purpose

For the intra-band non-contiguous carrier aggregation, the Error Vector Magnitude requirement should be defined for each component carrier. Requirement applies for the allocated component carrier, when all other component carriers are activated, but not allocated.

Similar transmitter impairment removal procedures are applied for CA waveform before EVM calculation as is specified for non-CA waveform in sub-section 6.5.2.1.

6.5.2A.1.3.2	Test applicability

This test case applies to all types of E-UTRA UE Release 11 and forward that support intra-band non-contiguous DL CA and UL CA.

6.5.2A.1.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5.2A.1.0.

6.5.2A.1.3.4	Test description

6.5.2A.1.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on E-UTRA CA configurations specified in table 5.4.2A.1-3. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 6.5.2A.1.3.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.2A.1.3.4.1-1: Test Configuration

| Table Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Low Range for CC1; High Range for CC2 PCC-SCC: CC1-CC2 and CC2-CC1 |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  | Refer to test point Test only test points with Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  | Wgap [MHz] | DL Allocation |  | UL Allocation |  |  |  |
|  | PCC NRB | SCCs NRB |  | CC MOD | PCC & SCC RB allocation | CC MOD | PCC NRB_alloc (LCRB @ RBstart) | CC MOD | SCC NRB_alloc (LCRB @ RBstart) |
| Test Parameters for CA_4A-4A Configurations |  |  |  |  |  |  |  |  |  |
| 1 | 25 | 25 | 35 | QPSK | N/A | QPSK | P_8@0 | QPSK | S_0@0 |
| 2 | 25 | 25 | 35 | QPSK |  | QPSK | P_8@17 | QPSK | S_0@0 |
| 3 | 25 | 25 | 35 | QPSK |  | QPSK | P_25@0 | QPSK | S_0@0 |
| 4 | 25 | 25 | 35 | 16QAM |  | 16QAM | P_8@0 | 16QAM | S_0@0 |
| 5 | 25 | 25 | 35 | 16QAM |  | 16QAM | P_8@17 | 16QAM | S_0@0 |
| 6 | 25 | 25 | 35 | 16QAM |  | 16QAM | P_25@0 | 16QAM | S_0@0 |
| 7 | 50 | 50 | 25 | QPSK |  | QPSK | P_12@0 | QPSK | S_0@0 |
| 8 | 50 | 50 | 25 | QPSK |  | QPSK | P_12@38 | QPSK | S_0@0 |
| 9 | 50 | 50 | 25 | QPSK |  | QPSK | P_50@0 | QPSK | S_0@0 |
| 10 | 50 | 50 | 25 | 16QAM |  | 16QAM | P_12@0 | 16QAM | S_0@0 |
| 11 | 50 | 50 | 25 | 16QAM |  | 16QAM | P_12@38 | 16QAM | S_0@0 |
| 12 | 50 | 50 | 25 | 16QAM |  | 16QAM | P_50@0 | 16QAM | S_0@0 |
| 13 | 75 | 75 | 15 | QPSK |  | QPSK | P_16@0 | QPSK | S_0@0 |
| 14 | 75 | 75 | 15 | QPSK |  | QPSK | P_16@59 | QPSK | S_0@0 |
| 15 | 75 | 75 | 15 | QPSK |  | QPSK | P_75@0 | QPSK | S_0@0 |
| 16 | 75 | 75 | 15 | 16QAM |  | 16QAM | P_16@0 | 16QAM | S_0@0 |
| 17 | 75 | 75 | 15 | 16QAM |  | 16QAM | P_16@59 | 16QAM | S_0@0 |
| 18 | 75 | 75 | 15 | 16QAM |  | 16QAM | P_75@0 | 16QAM | S_0@0 |
| 19 | 100 | 100 | 5 | QPSK |  | QPSK | P_18@0 | QPSK | S_0@0 |
| 20 | 100 | 100 | 5 | QPSK |  | QPSK | P_18@82 | QPSK | S_0@0 |
| 21 | 100 | 100 | 5 | QPSK |  | QPSK | P_100@0 | QPSK | S_0@0 |
| 22 | 100 | 100 | 5 | 16QAM |  | 16QAM | P_18@0 | 16QAM | S_0@0 |
| 23 | 100 | 100 | 5 | 16QAM |  | 16QAM | P_18@82 | 16QAM | S_0@0 |
| 24 | 100 | 100 | 5 | 16QAM |  | 16QAM | P_100@0 | 16QAM | S_0@0 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-3. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL RMC is set according to Table 6.5.2A.1.3.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.5.2A.1.3.4.3.

6.5.2A.1.3.4.2	Test procedure

Test procedures should be as follows:

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.5.2A.3.2.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.5.2A.1.3.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

6.	Measure the EVM and  on PCC using Global In-Channel Tx-Test (Annex E).

7.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is –36.8dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -36.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

8.	Measure the EVM and  on PCC using Global In-Channel Tx-Test (Annex E).

6.5.2A.1.3.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

6.5.2A.1.3.5	Test requirement

The PUSCH EVM derived in Annex E.4.2 shall not exceed 17,5 % for QPSK and BPSK, 12,5% for 16 QAM. The PUSCH derived in Annex E.4.6.2 shall not exceed 17,5 % when embedded with data symbols of QPSK and BPSK, 12,5% for 16 QAM.

##### 6.5.2A.1.3_1	Error Vector Magnitude (EVM) for CA (intra-band non-contiguous DL CA and UL CA) for UL 64QAM

6.5.2A.1.3_1.1	Test Purpose

For the intra-band non-contiguous carrier aggregation, the Error Vector Magnitude requirement should be defined for each component carrier. Requirement applies for the allocated component carrier, when all other component carriers are activated, but not allocated.

Similar transmitter impairment removal procedures are applied for CA waveform before EVM calculation as is specified for non-CA waveform in sub-section 6.5.2.1.

6.5.2A.1.3_1.2	Test applicability

This test case applies to all types of E-UTRA UE Release 13 and forward that support intra-band non-contiguous DL CA and UL CA and UL 64QAM.

6.5.2A.1.3_1.3	Minimum conformance requirements

The EVM requirement for 64QAM is according to Table 6.5.2A.1.1_1.3-1 if CA is configured in uplink.

Table 6.5.2A.1.1_1.3-1: Minimum requirements for Error Vector Magnitude

| Parameter | Unit | Average EVM Level per CC | Reference Signal EVM Level |
| --- | --- | --- | --- |
| 64QAM | % | 8 | 8 |

6.5.2A.1.3_1.4	Test description

6.5.2A.1.3_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on E-UTRA CA configurations specified in table 5.4.2A.1-3. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 6.5.2A.1.3_1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.2A.1.3_1.4.1-1: Test Configuration

| Table Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Low Range for CC1; High Range for CC2 PCC-SCC: CC1-CC2 and CC2-CC1 |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  | Refer to test point Test only test points with Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  | Wgap [MHz] | DL Allocation |  | UL Allocation |  |  |  |
|  | PCC NRB | SCCs NRB |  | CC MOD | PCC & SCC RB allocation | CC MOD | PCC NRB_alloc (LCRB @ RBstart) | CC MOD | SCC NRB_alloc (LCRB @ RBstart) |
| 1 | 25 | 25 | 35 | 64QAM | N/A | 64QAM | P_8@0 | 64QAM | S_0@0 |
| 2 | 25 | 25 | 35 | 64QAM |  | 64QAM | P_8@17 | 64QAM | S_0@0 |
| 3 | 25 | 25 | 35 | 64QAM |  | 64QAM | P_25@0 | 64QAM | S_0@0 |
| 4 | 50 | 50 | 25 | 64QAM |  | 64QAM | P_12@0 | 64QAM | S_0@0 |
| 5 | 50 | 50 | 25 | 64QAM |  | 64QAM | P_12@38 | 64QAM | S_0@0 |
| 6 | 50 | 50 | 25 | 64QAM |  | 64QAM | P_50@0 | 64QAM | S_0@0 |
| 7 | 75 | 75 | 15 | 64QAM |  | 64QAM | P_16@0 | 64QAM | S_0@0 |
| 8 | 75 | 75 | 15 | 64QAM |  | 64QAM | P_16@59 | 64QAM | S_0@0 |
| 9 | 75 | 75 | 15 | 64QAM |  | 64QAM | P_75@0 | 64QAM | S_0@0 |
| 10 | 100 | 100 | 5 | 64QAM |  | 64QAM | P_18@0 | 64QAM | S_0@0 |
| 11 | 100 | 100 | 5 | 64QAM |  | 64QAM | P_18@82 | 64QAM | S_0@0 |
| 12 | 100 | 100 | 5 | 64QAM |  | 64QAM | P_100@0 | 64QAM | S_0@0 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-3. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL RMC is set according to Table 6.5.2A.1.3_1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.5.2A.1.3_1.4.3.

6.5.2A.1.3_1.4.2	Test procedure

Same test procedure as in clause 6.5.2A.1.3.4.2.

6.5.2A.1.3_1.4.3	Message contents

Same message contents as in clause 6.5.2A.1.3.4.3.

6.5.2A.1.3_1.5	Test requirement

The PUSCH EVM derived in Annex E.4.2 shall not exceed 8 % for 64QAM. The PUSCH derived in Annex E.4.6.2 shall not exceed 8 % when embedded with data symbols of 64QAM.

##### 6.5.2A.1.3_2	Error Vector Magnitude (EVM) for CA (intra-band non-contiguous DL CA and UL CA) for UL 256QAM

6.5.2A.1.3_2.1	Test Purpose

For the intra-band non-contiguous carrier aggregation, the Error Vector Magnitude requirement should be defined for each component carrier. Requirement applies for the allocated component carrier, when all other component carriers are activated, but not allocated.

Similar transmitter impairment removal procedures are applied for CA waveform before EVM calculation as is specified for non-CA waveform in sub-section 6.5.2.1.

6.5.2A.1.3_2.2	Test applicability

This test case applies to all types of E-UTRA UE release 14 and forward that support intra-band non-contiguous DL CA and UL CA and UL 256QAM.

6.5.2A.1.3_2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5.2A.1.0

6.5.2A.1.3_2.4	Test description

6.5.2A.1.3_2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on E-UTRA CA configurations specified in table 5.4.2A.1-3. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 6.5.2A.1.3_2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.2A.1.3_2.4.1-1: Test Configuration

| Table Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Low Range for CC1; High Range for CC2 PCC-SCC: CC1-CC2 and CC2-CC1 |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  | Refer to test point Test only test points with Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  | Wgap [MHz] | DL Allocation |  | UL Allocation |  |  |  |
|  | PCC NRB Note 1 | SCCs NRB Note 1 |  | PCC & SCC RB allocation |  | CC MOD | PCC NRB_alloc (LCRB @ RBstart) | CC MOD | SCC NRB_alloc (LCRB @ RBstart) |
| Test Parameters for CA_4A-4A Configurations |  |  |  |  |  |  |  |  |  |
| 1 | 25 | 25 | 35 | N/A |  | 256QAM | P_25@0 | 256QAM | S_0@0 |
| 2 | 50 | 50 | 25 |  |  | 256QAM | P_50@0 | 256QAM | S_0@0 |
| 3 | 75 | 75 | 15 |  |  | 256QAM | P_75@0 | 256QAM | S_0@0 |
| 4 | 100 | 100 | 5 |  |  | 256QAM | P_100@0 | 256QAM | S_0@0 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-3. |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL RMC is set according to Table 6.5.2A.1.3_2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.5.2A.1.3_2.4.3.

6.5.2A.1.3_2.4.2	Test procedure

Same as in clause 6.5.2A.1.3.4.2 with the following exceptions:

-	Instead of Table 6.5.2A.1.3.4.1-1  use Table 6.5.2A.1.3_2.4.1-1.

-	Instead of UE output power -36.8dBm ±3.2dB  -26.8dBm ±3.2dB for carrier frequency f ≤ 3.0GHz.

-	Instead of UE output power -36.5dBm ±3.5dB  -26.5dBm ±3.5dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

6.5.2A.1.3_2.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

6.5.2A.1.3_2.5	Test requirement

The PUSCH EVM derived in Annex E.4.2 shall not exceed the values in Table 6.5.2.1_2.5-1 for 256QAM.

The PUSCH derived in Annex E.4.6.2 shall not exceed the values in Table 6.5.2.1_2.5-1 when embedded with data symbols of 256QAM.

##### 6.5.2A.1.4	Error Vector Magnitude (EVM) for CA (3UL CA)

6.5.2A.1.4.1	Test Purpose

The Error Vector Magnitude requirement should be defined for each component carrier. Requirement applies for the allocated component carrier, when all other component carriers are activated, but not allocated.

Similar transmitter impairment removal procedures are applied for CA waveform before EVM calculation as is specified for non-CA waveform in sub-section 6.5.2.1.

6.5.2A.1.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 13 and forward that support intra-band contiguous or inter-band 3DL CA and 3UL CA.

6.5.2A.1.4.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5.2A.1.0.

6.5.2A.1.4.4	Test description

6.5.2A.1.4.4.1		Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on E-UTRA CA configurations specified in table 5.4.2A.1-2 for CA configurations with two bands. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 6.5.2A.1.4.4.1-1 to 6.5.2A.1.4.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.2A.1.4.4.1-1: Test Configuration Table for intra-band contiguous CA

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Low range, High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  | Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |
|  | PCC NRB Note 1 | SCCs NRB Note 1 | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC1 & SCC2 RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 50 | 50 | N/A | QPSK | 50 | P_50@0 | S_0@0 | S_0@0 | - |
| 2 | 50 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 | - |
| 3 | 50 | 50 |  | QPSK | 12 | P_12@38 | S_0@0 | S_0@0 | - |
| 4 | 50 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 | S_0@0 | - |
| 5 | 50 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | S_0@0 | - |
| 6 | 50 | 50 |  | 16QAM | 12 | P_12@38 | S_0@0 | S_0@0 | - |
| 7 | 50 | 25 |  | QPSK | 50 | P_50@0 | S_0@0 | S_0@0 | - |
| 8 | 50 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | S_0@0 | - |
| 9 | 50 | 25 |  | QPSK | 25 | P_25@25 | S_0@0 | S_0@0 | - |
| 10 | 50 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | - |
| 11 | 50 | 25 |  | QPSK | 8 | P_8@42 | S_0@0 | S_0@0 | - |
| 12 | 50 | 25 |  | 16QAM | 50 | P_50@0 | S_0@0 | S_0@0 | - |
| 13 | 50 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | S_0@0 | - |
| 14 | 50 | 25 |  | 16QAM | 25 | P_25@25 | S_0@0 | S_0@0 | - |
| 15 | 50 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | S_0@0 | - |
| 16 | 50 | 25 |  | 16QAM | 8 | P_8@42 | S_0@0 | S_0@0 | - |
| 17 | 25 | 50 |  | QPSK | 25 | P_25@0 | S_0@0 | S_0@0 | - |
| 18 | 25 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 | - |
| 19 | 25 | 50 |  | QPSK | 12 | P_12@13 | S_0@0 | S_0@0 | - |
| 20 | 25 | 50 |  | 16QAM | 25 | P_25@0 | S_0@0 | S_0@0 | - |
| 21 | 25 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | S_0@0 | - |
| 22 | 25 | 50 |  | 16QAM | 12 | P_12@13 | S_0@0 | S_0@0 | - |
| 23 | 75 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 | S_0@0 | - |
| 24 | 75 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | S_0@0 | - |
| 25 | 75 | 75 |  | QPSK | 16 | P_16@59 | S_0@0 | S_0@0 | - |
| 26 | 75 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 | S_0@0 | - |
| 27 | 75 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 | S_0@0 | - |
| 28 | 75 | 75 |  | 16QAM | 16 | P_16@59 | S_0@0 | S_0@0 | - |
| 29 | 100 | 25 |  | QPSK | 100 | P_100@0 | S_0@0 | S_0@0 | - |
| 30 | 100 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | S_0@0 | - |
| 31 | 100 | 25 |  | QPSK | 25 | P_25@75 | S_0@0 | S_0@0 | - |
| 32 | 100 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | - |
| 33 | 100 | 25 |  | QPSK | 8 | P_8@92 | S_0@0 | S_0@0 | - |
| 34 | 100 | 25 |  | 16QAM | 100 | P_100@0 | S_0@0 | S_0@0 | - |
| 35 | 100 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | S_0@0 | - |
| 36 | 100 | 25 |  | 16QAM | 25 | P_25@75 | S_0@0 | S_0@0 | - |
| 37 | 100 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | S_0@0 | - |
| 38 | 100 | 25 |  | 16QAM | 8 | P_8@92 | S_0@0 | S_0@0 | - |
| 39 | 25 | 100 |  | QPSK | 25 | P_25@0 | S_0@0 | S_0@0 | - |
| 40 | 25 | 100 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | - |
| 41 | 25 | 100 |  | QPSK | 8 | P_8@17 | S_0@0 | S_0@0 | - |
| 42 | 25 | 100 |  | 16QAM | 25 | P_25@0 | S_0@0 | S_0@0 | - |
| 43 | 25 | 100 |  | 16QAM | 8 | P_8@0 | S_0@0 | S_0@0 | - |
| 44 | 25 | 100 |  | 16QAM | 8 | P_8@17 | S_0@0 | S_0@0 | - |
| 45 | 100 | 50 |  | QPSK | 100 | P_100@0 | S_0@0 | S_0@0 | - |
| 46 | 100 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | S_0@0 | - |
| 47 | 100 | 50 |  | QPSK | 50 | P_50@50 | S_0@0 | S_0@0 | - |
| 48 | 100 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 | - |
| 49 | 100 | 50 |  | QPSK | 12 | P_12@88 | S_0@0 | S_0@0 | - |
| 50 | 100 | 50 |  | 16QAM | 100 | P_100@0 | S_0@0 | S_0@0 | - |
| 51 | 100 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 | S_0@0 | - |
| 52 | 100 | 50 |  | 16QAM | 50 | P_50@50 | S_0@0 | S_0@0 | - |
| 53 | 100 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | S_0@0 | - |
| 54 | 100 | 50 |  | 16QAM | 12 | P_12@88 | S_0@0 | S_0@0 | - |
| 55 | 50 | 100 |  | QPSK | 50 | P_50@0 | S_0@0 | S_0@0 | - |
| 56 | 50 | 100 |  | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 | - |
| 57 | 50 | 100 |  | QPSK | 12 | P_12@38 | S_0@0 | S_0@0 | - |
| 58 | 50 | 100 |  | 16QAM | 50 | P_50@0 | S_0@0 | S_0@0 | - |
| 59 | 50 | 100 |  | 16QAM | 12 | P_12@0 | S_0@0 | S_0@0 | - |
| 60 | 50 | 100 |  | 16QAM | 12 | P_12@38 | S_0@0 | S_0@0 | - |
| 61 | 100 | 75 |  | QPSK | 100 | P_100@0 | S_0@0 | S_0@0 | - |
| 62 | 100 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 | S_0@0 | - |
| 63 | 100 | 75 |  | QPSK | 75 | P_75@25 | S_0@0 | S_0@0 | - |
| 64 | 100 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | S_0@0 | - |
| 65 | 100 | 75 |  | QPSK | 16 | P_16@84 | S_0@0 | S_0@0 | - |
| 66 | 100 | 75 |  | 16QAM | 100 | P_100@0 | S_0@0 | S_0@0 | - |
| 67 | 100 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 | S_0@0 | - |
| 68 | 100 | 75 |  | 16QAM | 75 | P_75@25 | S_0@0 | S_0@0 | - |
| 69 | 100 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 | S_0@0 | - |
| 70 | 100 | 75 |  | 16QAM | 16 | P_16@84 | S_0@0 | S_0@0 | - |
| 71 | 75 | 100 |  | QPSK | 75 | P_75@0 | S_0@0 | S_0@0 | - |
| 72 | 75 | 100 |  | QPSK | 16 | P_16@0 | S_0@0 | S_0@0 | - |
| 73 | 75 | 100 |  | QPSK | 16 | P_16@59 | S_0@0 | S_0@0 | - |
| 74 | 75 | 100 |  | 16QAM | 75 | P_75@0 | S_0@0 | S_0@0 | - |
| 75 | 75 | 100 |  | 16QAM | 16 | P_16@0 | S_0@0 | S_0@0 | - |
| 76 | 75 | 100 |  | 16QAM | 16 | P_16@59 | S_0@0 | S_0@0 | - |
| 77 | 100 | 100 |  | QPSK | 100 | P_100@0 | S_0@0 | S_0@0 | - |
| 78 | 100 | 100 |  | QPSK | 18 | P_18@0 | S_0@0 | S_0@0 | - |
| 79 | 100 | 100 |  | QPSK | 18 | P_18@82 | S_0@0 | S_0@0 | - |
| 80 | 100 | 100 |  | 16QAM | 100 | P_100@0 | S_0@0 | S_0@0 | - |
| 81 | 100 | 100 |  | 16QAM | 18 | P_18@0 | S_0@0 | S_0@0 | - |
| 82 | 100 | 100 |  | 16QAM | 18 | P_18@82 | S_0@0 | S_0@0 | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |  |

Table 6.5.2A.1.4.4.1-2: Test Configuration Table for inter-band CA

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  |  | Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |
| Config ID |  | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |
|  |  | PCC NRB Note 1 | SCCs NRB Note 1 | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC1 & SCC2 RB allocations (LCRB @ RBstart) |  |  |  |
| 1 |  | 6 | 25 | N/A | QPSK | 1 | P_1@0 | S_0@0 | S_0@0 | - |
| 2 |  | 6 | 25 |  | QPSK | 1 | P_1@5 | S_0@0 | S_0@0 | - |
| 3 |  | 6 | 25 |  | QPSK | 6 | P_6@0 | S_0@0 | S_0@0 | - |
| 4 |  | 6 | 25 |  | 16QAM | 1 | P_1@0 | S_0@0 | S_0@0 | - |
| 5 |  | 6 | 25 |  | 16QAM | 1 | P_1@5 | S_0@0 | S_0@0 | - |
| 6 |  | 6 | 25 |  | 16QAM | 6 | P_6@0 | S_0@0 | S_0@0 | - |
| 7 |  | 6 | 50 |  | QPSK | 1 | P_1@0 | S_0@0 | S_0@0 | - |
| 8 |  | 6 | 50 |  | QPSK | 1 | P_1@5 | S_0@0 | S_0@0 | - |
| 9 |  | 6 | 50 |  | QPSK | 6 | P_6@0 | S_0@0 | S_0@0 | - |
| 10 |  | 6 | 50 |  | 16QAM | 1 | P_1@0 | S_0@0 | S_0@0 | - |
| 11 |  | 6 | 50 |  | 16QAM | 1 | P_1@5 | S_0@0 | S_0@0 | - |
| 12 |  | 6 | 50 |  | 16QAM | 6 | P_6@0 | S_0@0 | S_0@0 | - |
| 13 |  | 15 | 25 |  | QPSK | 4 | P_4@0 | S_0@0 | S_0@0 | - |
| 14 |  | 15 | 25 |  | QPSK | 4 | P_4@11 | S_0@0 | S_0@0 | - |
| 15 |  | 15 | 25 |  | QPSK | 15 | P_15@0 | S_0@0 | S_0@0 | - |
| 16 |  | 15 | 25 |  | 16QAM | 4 | P_4@0 | S_0@0 | S_0@0 | - |
| 17 |  | 15 | 25 |  | 16QAM | 4 | P_4@11 | S_0@0 | S_0@0 | - |
| 18 |  | 15 | 25 |  | 16QAM | 15 | P_15@0 | S_0@0 | S_0@0 | - |
| 19 |  | 25 | 6 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | - |
| 20 |  | 25 | 6 |  | QPSK | 8 | P_8@17 | S_0@0 | S_0@0 | - |
| 21 |  | 25 | 6 |  | QPSK | 25 | P_25@0 | S_0@0 | S_0@0 | - |
| 22 |  | 25 | 6 |  | 16QAM | 8 | P_8@0 | S_0@0 | S_0@0 | - |
| 23 |  | 25 | 6 |  | 16QAM | 8 | P_8@17 | S_0@0 | S_0@0 | - |
| 24 |  | 25 | 6 |  | 16QAM | 25 | P_25@0 | S_0@0 | S_0@0 | - |
| 25 |  | 25 | 15 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | - |
| 26 |  | 25 | 15 |  | QPSK | 8 | P_8@17 | S_0@0 | S_0@0 | - |
| 27 |  | 25 | 15 |  | QPSK | 25 | P_25@0 | S_0@0 | S_0@0 | - |
| 28 |  | 25 | 15 |  | 16QAM | 8 | P_8@0 | S_0@0 | S_0@0 | - |
| 29 |  | 25 | 15 |  | 16QAM | 8 | P_8@17 | S_0@0 | S_0@0 | - |
| 30 |  | 25 | 15 |  | 16QAM | 25 | P_25@0 | S_0@0 | S_0@0 | - |
| 31 | 25 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | - |
| 32 | 25 | 25 |  | QPSK | 8 | P_8@17 | S_0@0 | S_0@0 | - |
| 33 | 25 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | S_0@0 | - |
| 34 | 25 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | S_0@0 | - |
| 35 | 25 | 25 |  | 16QAM | 8 | P_8@17 | S_0@0 | S_0@0 | - |
| 36 | 25 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | S_0@0 | - |
| 37 | 25 | 50 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | - |
| 38 | 25 | 50 |  | QPSK | 8 | P_8@17 | S_0@0 | S_0@0 | - |
| 39 | 25 | 50 |  | QPSK | 25 | P_25@0 | S_0@0 | S_0@0 | - |
| 40 | 25 | 50 |  | 16QAM | 8 | P_8@0 | S_0@0 | S_0@0 | - |
| 41 | 25 | 50 |  | 16QAM | 8 | P_8@17 | S_0@0 | S_0@0 | - |
| 42 | 25 | 50 |  | 16QAM | 25 | P_25@0 | S_0@0 | S_0@0 | - |
| 43 | 50 | 6 |  | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 | - |
| 44 | 50 | 6 |  | QPSK | 12 | P_12@38 | S_0@0 | S_0@0 | - |
| 45 | 50 | 6 |  | QPSK | 50 | P_50@0 | S_0@0 | S_0@0 | - |
| 46 | 50 | 6 |  | 16QAM | 12 | P_12@0 | S_0@0 | S_0@0 | - |
| 47 | 50 | 6 |  | 16QAM | 12 | P_12@38 | S_0@0 | S_0@0 | - |
| 48 | 50 | 6 |  | 16QAM | 50 | P_50@0 | S_0@0 | S_0@0 | - |
| 49 | 50 | 25 |  | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 | - |
| 50 | 50 | 25 |  | QPSK | 12 | P_12@38 | S_0@0 | S_0@0 | - |
| 51 | 50 | 25 |  | QPSK | 50 | P_50@0 | S_0@0 | S_0@0 | - |
| 52 | 50 | 25 |  | 16QAM | 12 | P_12@0 | S_0@0 | S_0@0 | - |
| 53 | 50 | 25 |  | 16QAM | 12 | P_12@38 | S_0@0 | S_0@0 | - |
| 54 | 50 | 25 |  | 16QAM | 50 | P_50@0 | S_0@0 | S_0@0 | - |
| 55 | 50 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 | - |
| 56 | 50 | 50 |  | QPSK | 12 | P_12@38 | S_0@0 | S_0@0 | - |
| 57 | 50 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | S_0@0 | - |
| 58 | 50 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | S_0@0 | - |
| 59 | 50 | 50 |  | 16QAM | 12 | P_12@38 | S_0@0 | S_0@0 | - |
| 60 | 50 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 | S_0@0 | - |
| 61 | 50 | 100 |  | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 | - |
| 62 | 50 | 100 |  | QPSK | 12 | P_12@38 | S_0@0 | S_0@0 | - |
| 63 | 50 | 100 |  | QPSK | 50 | P_50@0 | S_0@0 | S_0@0 | - |
| 64 | 50 | 100 |  | 16QAM | 12 | P_12@0 | S_0@0 | S_0@0 | - |
| 65 | 50 | 100 |  | 16QAM | 12 | P_12@38 | S_0@0 | S_0@0 | - |
| 66 | 50 | 100 |  | 16QAM | 50 | P_50@0 | S_0@0 | S_0@0 | - |
| 67 | 75 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | S_0@0 | - |
| 68 | 75 | 75 |  | QPSK | 16 | P_16@59 | S_0@0 | S_0@0 | - |
| 69 | 75 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 | S_0@0 | - |
| 70 | 75 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 | S_0@0 | - |
| 71 | 75 | 75 |  | 16QAM | 16 | P_16@59 | S_0@0 | S_0@0 | - |
| 72 | 75 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 | S_0@0 | - |
| 73 | 75 | 100 |  | QPSK | 16 | P_16@0 | S_0@0 | S_0@0 | - |
| 74 | 75 | 100 |  | QPSK | 16 | P_16@59 | S_0@0 | S_0@0 | - |
| 75 | 75 | 100 |  | QPSK | 75 | P_75@0 | S_0@0 | S_0@0 | - |
| 76 | 75 | 100 |  | 16QAM | 16 | P_16@0 | S_0@0 | S_0@0 | - |
| 77 | 75 | 100 |  | 16QAM | 16 | P_16@59 | S_0@0 | S_0@0 | - |
| 78 | 75 | 100 |  | 16QAM | 75 | P_75@0 | S_0@0 | S_0@0 | - |
| 79 | 100 | 50 |  | QPSK | 18 | P_18@0 | S_0@0 | S_0@0 | - |
| 80 | 100 | 50 |  | QPSK | 18 | P_18@82 | S_0@0 | S_0@0 | - |
| 81 | 100 | 50 |  | QPSK | 100 | P_100@0 | S_0@0 | S_0@0 | - |
| 82 | 100 | 50 |  | 16QAM | 18 | P_18@0 | S_0@0 | S_0@0 | - |
| 83 | 100 | 50 |  | 16QAM | 18 | P_18@82 | S_0@0 | S_0@0 | - |
| 84 | 100 | 50 |  | 16QAM | 100 | P_100@0 | S_0@0 | S_0@0 | - |
| 85 | 100 | 75 |  | QPSK | 18 | P_18@0 | S_0@0 | S_0@0 | - |
| 86 | 100 | 75 |  | QPSK | 18 | P_18@82 | S_0@0 | S_0@0 | - |
| 87 | 100 | 75 |  | QPSK | 100 | P_100@0 | S_0@0 | S_0@0 | - |
| 88 | 100 | 75 |  | 16QAM | 18 | P_18@0 | S_0@0 | S_0@0 | - |
| 89 | 100 | 75 |  | 16QAM | 18 | P_18@82 | S_0@0 | S_0@0 | - |
| 90 | 100 | 75 |  | 16QAM | 100 | P_100@0 | S_0@0 | S_0@0 | - |
| 91 | 100 | 100 |  | QPSK | 18 | P_18@0 | S_0@0 | S_0@0 | - |
| 92 | 100 | 100 |  | QPSK | 18 | P_18@82 | S_0@0 | S_0@0 | - |
| 93 | 100 | 100 |  | QPSK | 100 | P_100@0 | S_0@0 | S_0@0 | - |
| 94 | 100 | 100 |  | 16QAM | 18 | P_18@0 | S_0@0 | S_0@0 | - |
| 95 | 100 | 100 |  | 16QAM | 18 | P_18@82 | S_0@0 | S_0@0 | - |
| 96 | 100 | 100 |  | 16QAM | 100 | P_100@0 | S_0@0 | S_0@0 | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: The frequencies of PCC and SCCs shall be switched and tested in each configuration. |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A, Figure A.55 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.5.2A.1.4.4.1-1 to 6.5.2A.1.4.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.5.2A.1.4.4.3.

6.5.2A.1.4.4.2	Test procedure

1.	Configure SCCs according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCCs as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.5.2A.1.4.4.3.

3.	SS activates SCCs by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.5.2A.1.4.4.1-1 to 6.5.2A.1.4.4.1-2. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

6.	Measure the EVM and  on PCC using Global In-Channel Tx-Test (Annex E).

7.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is –36.8dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -36.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

8.	Measure the EVM and  on PCC using Global In-Channel Tx-Test (Annex E).

6.5.2A.1.4.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCCs configuration there are no additional message contents.

6.5.2A.1.4.5	Test requirement

The PUSCH EVM derived in E.4.2 shall not exceed 17,5 % for QPSK and BPSK, 12,5% for 16 QAM.

The PUSCH derived in E.4.6.2 shall not exceed 17,5 % when embedded with data symbols of QPSK and BPSK, 12,5% for 16 QAM.

##### 6.5.2A.1.5	Error Vector Magnitude (EVM) for CA (4UL CA)

6.5.2A.1.5.1	Test Purpose

The Error Vector Magnitude requirement should be defined for each component carrier. Requirement applies for the allocated component carrier, when all other component carriers are activated, but not allocated.

Similar transmitter impairment removal procedures are applied for CA waveform before EVM calculation as is specified for non-CA waveform in sub-section 6.5.2.1.

6.5.2A.1.5.2	Test applicability

This test case applies to all types of E-UTRA UE release 11 and forward that support 4DL CA and 4UL CA.

6.5.2A.1.5.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5.2A.1.0.

6.5.2A.1.5.4	Test description

6.5.2A.1.5.4.1		Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on E-UTRA CA configurations specified in table 5.4.2A.1. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 6.5.2A.1.5.4.1-1 to 6.5.2A.1.5.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.2A.1.5.4.1-1: Test Configuration Table for intra-band contiguous CA

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Low range, High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  | Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |
|  | PCC NRB Note 1 | SCCs NRB Note 1 | PCC & SCCs RB allocation |  | NRB_alloc | PCC & SCC1 & SCC2 & SCC3 RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 50 | 50 | N/A | QPSK | 50 | P_50@0 | S_0@0 | S_0@0 | S_0@0 |
| 2 | 50 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 | S_0@0 |
| 3 | 50 | 50 |  | QPSK | 12 | P_12@38 | S_0@0 | S_0@0 | S_0@0 |
| 4 | 50 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 | S_0@0 | S_0@0 |
| 5 | 50 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | S_0@0 | S_0@0 |
| 6 | 50 | 50 |  | 16QAM | 12 | P_12@38 | S_0@0 | S_0@0 | S_0@0 |
| 7 | 50 | 25 |  | QPSK | 50 | P_50@0 | S_0@0 | S_0@0 | S_0@0 |
| 8 | 50 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | S_0@0 | S_0@0 |
| 9 | 50 | 25 |  | QPSK | 25 | P_25@25 | S_0@0 | S_0@0 | S_0@0 |
| 10 | 50 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | S_0@0 |
| 11 | 50 | 25 |  | QPSK | 8 | P_8@42 | S_0@0 | S_0@0 | S_0@0 |
| 12 | 50 | 25 |  | 16QAM | 50 | P_50@0 | S_0@0 | S_0@0 | S_0@0 |
| 13 | 50 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | S_0@0 | S_0@0 |
| 14 | 50 | 25 |  | 16QAM | 25 | P_25@25 | S_0@0 | S_0@0 | S_0@0 |
| 15 | 50 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | S_0@0 | S_0@0 |
| 16 | 50 | 25 |  | 16QAM | 8 | P_8@42 | S_0@0 | S_0@0 | S_0@0 |
| 17 | 25 | 50 |  | QPSK | 25 | P_25@0 | S_0@0 | S_0@0 | S_0@0 |
| 18 | 25 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 | S_0@0 |
| 19 | 25 | 50 |  | QPSK | 12 | P_12@13 | S_0@0 | S_0@0 | S_0@0 |
| 20 | 25 | 50 |  | 16QAM | 25 | P_25@0 | S_0@0 | S_0@0 | S_0@0 |
| 21 | 25 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | S_0@0 | S_0@0 |
| 22 | 25 | 50 |  | 16QAM | 12 | P_12@13 | S_0@0 | S_0@0 | S_0@0 |
| 23 | 75 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 | S_0@0 | S_0@0 |
| 24 | 75 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | S_0@0 | S_0@0 |
| 25 | 75 | 75 |  | QPSK | 16 | P_16@59 | S_0@0 | S_0@0 | S_0@0 |
| 26 | 75 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 | S_0@0 | S_0@0 |
| 27 | 75 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 | S_0@0 | S_0@0 |
| 28 | 75 | 75 |  | 16QAM | 16 | P_16@59 | S_0@0 | S_0@0 | S_0@0 |
| 29 | 100 | 25 |  | QPSK | 100 | P_100@0 | S_0@0 | S_0@0 | S_0@0 |
| 30 | 100 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | S_0@0 | S_0@0 |
| 31 | 100 | 25 |  | QPSK | 25 | P_25@75 | S_0@0 | S_0@0 | S_0@0 |
| 32 | 100 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | S_0@0 |
| 33 | 100 | 25 |  | QPSK | 8 | P_8@92 | S_0@0 | S_0@0 | S_0@0 |
| 34 | 100 | 25 |  | 16QAM | 100 | P_100@0 | S_0@0 | S_0@0 | S_0@0 |
| 35 | 100 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | S_0@0 | S_0@0 |
| 36 | 100 | 25 |  | 16QAM | 25 | P_25@75 | S_0@0 | S_0@0 | S_0@0 |
| 37 | 100 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | S_0@0 | S_0@0 |
| 38 | 100 | 25 |  | 16QAM | 8 | P_8@92 | S_0@0 | S_0@0 | S_0@0 |
| 39 | 25 | 100 |  | QPSK | 25 | P_25@0 | S_0@0 | S_0@0 | S_0@0 |
| 40 | 25 | 100 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | S_0@0 |
| 41 | 25 | 100 |  | QPSK | 8 | P_8@17 | S_0@0 | S_0@0 | S_0@0 |
| 42 | 25 | 100 |  | 16QAM | 25 | P_25@0 | S_0@0 | S_0@0 | S_0@0 |
| 43 | 25 | 100 |  | 16QAM | 8 | P_8@0 | S_0@0 | S_0@0 | S_0@0 |
| 44 | 25 | 100 |  | 16QAM | 8 | P_8@17 | S_0@0 | S_0@0 | S_0@0 |
| 45 | 100 | 50 |  | QPSK | 100 | P_100@0 | S_0@0 | S_0@0 | S_0@0 |
| 46 | 100 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | S_0@0 | S_0@0 |
| 47 | 100 | 50 |  | QPSK | 50 | P_50@50 | S_0@0 | S_0@0 | S_0@0 |
| 48 | 100 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 | S_0@0 |
| 49 | 100 | 50 |  | QPSK | 12 | P_12@88 | S_0@0 | S_0@0 | S_0@0 |
| 50 | 100 | 50 |  | 16QAM | 100 | P_100@0 | S_0@0 | S_0@0 | S_0@0 |
| 51 | 100 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 | S_0@0 | S_0@0 |
| 52 | 100 | 50 |  | 16QAM | 50 | P_50@50 | S_0@0 | S_0@0 | S_0@0 |
| 53 | 100 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | S_0@0 | S_0@0 |
| 54 | 100 | 50 |  | 16QAM | 12 | P_12@88 | S_0@0 | S_0@0 | S_0@0 |
| 55 | 50 | 100 |  | QPSK | 50 | P_50@0 | S_0@0 | S_0@0 | S_0@0 |
| 56 | 50 | 100 |  | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 | S_0@0 |
| 57 | 50 | 100 |  | QPSK | 12 | P_12@38 | S_0@0 | S_0@0 | S_0@0 |
| 58 | 50 | 100 |  | 16QAM | 50 | P_50@0 | S_0@0 | S_0@0 | S_0@0 |
| 59 | 50 | 100 |  | 16QAM | 12 | P_12@0 | S_0@0 | S_0@0 | S_0@0 |
| 60 | 50 | 100 |  | 16QAM | 12 | P_12@38 | S_0@0 | S_0@0 | S_0@0 |
| 61 | 100 | 75 |  | QPSK | 100 | P_100@0 | S_0@0 | S_0@0 | S_0@0 |
| 62 | 100 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 | S_0@0 | S_0@0 |
| 63 | 100 | 75 |  | QPSK | 75 | P_75@25 | S_0@0 | S_0@0 | S_0@0 |
| 64 | 100 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | S_0@0 | S_0@0 |
| 65 | 100 | 75 |  | QPSK | 16 | P_16@84 | S_0@0 | S_0@0 | S_0@0 |
| 66 | 100 | 75 |  | 16QAM | 100 | P_100@0 | S_0@0 | S_0@0 | S_0@0 |
| 67 | 100 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 | S_0@0 | S_0@0 |
| 68 | 100 | 75 |  | 16QAM | 75 | P_75@25 | S_0@0 | S_0@0 | S_0@0 |
| 69 | 100 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 | S_0@0 | S_0@0 |
| 70 | 100 | 75 |  | 16QAM | 16 | P_16@84 | S_0@0 | S_0@0 | S_0@0 |
| 71 | 75 | 100 |  | QPSK | 75 | P_75@0 | S_0@0 | S_0@0 | S_0@0 |
| 72 | 75 | 100 |  | QPSK | 16 | P_16@0 | S_0@0 | S_0@0 | S_0@0 |
| 73 | 75 | 100 |  | QPSK | 16 | P_16@59 | S_0@0 | S_0@0 | S_0@0 |
| 74 | 75 | 100 |  | 16QAM | 75 | P_75@0 | S_0@0 | S_0@0 | S_0@0 |
| 75 | 75 | 100 |  | 16QAM | 16 | P_16@0 | S_0@0 | S_0@0 | S_0@0 |
| 76 | 75 | 100 |  | 16QAM | 16 | P_16@59 | S_0@0 | S_0@0 | S_0@0 |
| 77 | 100 | 100 |  | QPSK | 100 | P_100@0 | S_0@0 | S_0@0 | S_0@0 |
| 78 | 100 | 100 |  | QPSK | 18 | P_18@0 | S_0@0 | S_0@0 | S_0@0 |
| 79 | 100 | 100 |  | QPSK | 18 | P_18@82 | S_0@0 | S_0@0 | S_0@0 |
| 80 | 100 | 100 |  | 16QAM | 100 | P_100@0 | S_0@0 | S_0@0 | S_0@0 |
| 81 | 100 | 100 |  | 16QAM | 18 | P_18@0 | S_0@0 | S_0@0 | S_0@0 |
| 82 | 100 | 100 |  | 16QAM | 18 | P_18@82 | S_0@0 | S_0@0 | S_0@0 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |  |

Table 6.5.2A.1.5.4.1-2: Test Configuration Table for inter-band CA

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  |  | Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |
| Config ID |  | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |
|  |  | PCC NRB Note 1 | SCCs NRB Note 1 | PCC & SCCs RB allocation |  | NRB_alloc | PCC & SCC1 & SCC2 & SCC3 RB allocations (LCRB @ RBstart) |  |  |  |
| 1 |  | 6 | 25 | N/A | QPSK | 1 | P_1@0 | S_0@0 | S_0@0 | S_0@0 |
| 2 |  | 6 | 25 |  | QPSK | 1 | P_1@5 | S_0@0 | S_0@0 | S_0@0 |
| 3 |  | 6 | 25 |  | QPSK | 6 | P_6@0 | S_0@0 | S_0@0 | S_0@0 |
| 4 |  | 6 | 25 |  | 16QAM | 1 | P_1@0 | S_0@0 | S_0@0 | S_0@0 |
| 5 |  | 6 | 25 |  | 16QAM | 1 | P_1@5 | S_0@0 | S_0@0 | S_0@0 |
| 6 |  | 6 | 25 |  | 16QAM | 6 | P_6@0 | S_0@0 | S_0@0 | S_0@0 |
| 7 |  | 6 | 50 |  | QPSK | 1 | P_1@0 | S_0@0 | S_0@0 | S_0@0 |
| 8 |  | 6 | 50 |  | QPSK | 1 | P_1@5 | S_0@0 | S_0@0 | S_0@0 |
| 9 |  | 6 | 50 |  | QPSK | 6 | P_6@0 | S_0@0 | S_0@0 | S_0@0 |
| 10 |  | 6 | 50 |  | 16QAM | 1 | P_1@0 | S_0@0 | S_0@0 | S_0@0 |
| 11 |  | 6 | 50 |  | 16QAM | 1 | P_1@5 | S_0@0 | S_0@0 | S_0@0 |
| 12 |  | 6 | 50 |  | 16QAM | 6 | P_6@0 | S_0@0 | S_0@0 | S_0@0 |
| 13 |  | 15 | 25 |  | QPSK | 4 | P_4@0 | S_0@0 | S_0@0 | S_0@0 |
| 14 |  | 15 | 25 |  | QPSK | 4 | P_4@11 | S_0@0 | S_0@0 | S_0@0 |
| 15 |  | 15 | 25 |  | QPSK | 15 | P_15@0 | S_0@0 | S_0@0 | S_0@0 |
| 16 |  | 15 | 25 |  | 16QAM | 4 | P_4@0 | S_0@0 | S_0@0 | S_0@0 |
| 17 |  | 15 | 25 |  | 16QAM | 4 | P_4@11 | S_0@0 | S_0@0 | S_0@0 |
| 18 |  | 15 | 25 |  | 16QAM | 15 | P_15@0 | S_0@0 | S_0@0 | S_0@0 |
| 19 |  | 25 | 6 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | S_0@0 |
| 20 |  | 25 | 6 |  | QPSK | 8 | P_8@17 | S_0@0 | S_0@0 | S_0@0 |
| 21 |  | 25 | 6 |  | QPSK | 25 | P_25@0 | S_0@0 | S_0@0 | S_0@0 |
| 22 |  | 25 | 6 |  | 16QAM | 8 | P_8@0 | S_0@0 | S_0@0 | S_0@0 |
| 23 |  | 25 | 6 |  | 16QAM | 8 | P_8@17 | S_0@0 | S_0@0 | S_0@0 |
| 24 |  | 25 | 6 |  | 16QAM | 25 | P_25@0 | S_0@0 | S_0@0 | S_0@0 |
| 25 |  | 25 | 15 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | S_0@0 |
| 26 |  | 25 | 15 |  | QPSK | 8 | P_8@17 | S_0@0 | S_0@0 | S_0@0 |
| 27 |  | 25 | 15 |  | QPSK | 25 | P_25@0 | S_0@0 | S_0@0 | S_0@0 |
| 28 |  | 25 | 15 |  | 16QAM | 8 | P_8@0 | S_0@0 | S_0@0 | S_0@0 |
| 29 |  | 25 | 15 |  | 16QAM | 8 | P_8@17 | S_0@0 | S_0@0 | S_0@0 |
| 30 |  | 25 | 15 |  | 16QAM | 25 | P_25@0 | S_0@0 | S_0@0 | S_0@0 |
| 31 | 25 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | S_0@0 |
| 32 | 25 | 25 |  | QPSK | 8 | P_8@17 | S_0@0 | S_0@0 | S_0@0 |
| 33 | 25 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | S_0@0 | S_0@0 |
| 34 | 25 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | S_0@0 | S_0@0 |
| 35 | 25 | 25 |  | 16QAM | 8 | P_8@17 | S_0@0 | S_0@0 | S_0@0 |
| 36 | 25 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | S_0@0 | S_0@0 |
| 37 | 25 | 50 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | S_0@0 |
| 38 | 25 | 50 |  | QPSK | 8 | P_8@17 | S_0@0 | S_0@0 | S_0@0 |
| 39 | 25 | 50 |  | QPSK | 25 | P_25@0 | S_0@0 | S_0@0 | S_0@0 |
| 40 | 25 | 50 |  | 16QAM | 8 | P_8@0 | S_0@0 | S_0@0 | S_0@0 |
| 41 | 25 | 50 |  | 16QAM | 8 | P_8@17 | S_0@0 | S_0@0 | S_0@0 |
| 42 | 25 | 50 |  | 16QAM | 25 | P_25@0 | S_0@0 | S_0@0 | S_0@0 |
| 43 | 50 | 6 |  | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 | S_0@0 |
| 44 | 50 | 6 |  | QPSK | 12 | P_12@38 | S_0@0 | S_0@0 | S_0@0 |
| 45 | 50 | 6 |  | QPSK | 50 | P_50@0 | S_0@0 | S_0@0 | S_0@0 |
| 46 | 50 | 6 |  | 16QAM | 12 | P_12@0 | S_0@0 | S_0@0 | S_0@0 |
| 47 | 50 | 6 |  | 16QAM | 12 | P_12@38 | S_0@0 | S_0@0 | S_0@0 |
| 48 | 50 | 6 |  | 16QAM | 50 | P_50@0 | S_0@0 | S_0@0 | S_0@0 |
| 49 | 50 | 25 |  | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 | S_0@0 |
| 50 | 50 | 25 |  | QPSK | 12 | P_12@38 | S_0@0 | S_0@0 | S_0@0 |
| 51 | 50 | 25 |  | QPSK | 50 | P_50@0 | S_0@0 | S_0@0 | S_0@0 |
| 52 | 50 | 25 |  | 16QAM | 12 | P_12@0 | S_0@0 | S_0@0 | S_0@0 |
| 53 | 50 | 25 |  | 16QAM | 12 | P_12@38 | S_0@0 | S_0@0 | S_0@0 |
| 54 | 50 | 25 |  | 16QAM | 50 | P_50@0 | S_0@0 | S_0@0 | S_0@0 |
| 55 | 50 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 | S_0@0 |
| 56 | 50 | 50 |  | QPSK | 12 | P_12@38 | S_0@0 | S_0@0 | S_0@0 |
| 57 | 50 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | S_0@0 | S_0@0 |
| 58 | 50 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | S_0@0 | S_0@0 |
| 59 | 50 | 50 |  | 16QAM | 12 | P_12@38 | S_0@0 | S_0@0 | S_0@0 |
| 60 | 50 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 | S_0@0 | S_0@0 |
| 61 | 50 | 100 |  | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 | S_0@0 |
| 62 | 50 | 100 |  | QPSK | 12 | P_12@38 | S_0@0 | S_0@0 | S_0@0 |
| 63 | 50 | 100 |  | QPSK | 50 | P_50@0 | S_0@0 | S_0@0 | S_0@0 |
| 64 | 50 | 100 |  | 16QAM | 12 | P_12@0 | S_0@0 | S_0@0 | S_0@0 |
| 65 | 50 | 100 |  | 16QAM | 12 | P_12@38 | S_0@0 | S_0@0 | S_0@0 |
| 66 | 50 | 100 |  | 16QAM | 50 | P_50@0 | S_0@0 | S_0@0 | S_0@0 |
| 67 | 75 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | S_0@0 | S_0@0 |
| 68 | 75 | 75 |  | QPSK | 16 | P_16@59 | S_0@0 | S_0@0 | S_0@0 |
| 69 | 75 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 | S_0@0 | S_0@0 |
| 70 | 75 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 | S_0@0 | S_0@0 |
| 71 | 75 | 75 |  | 16QAM | 16 | P_16@59 | S_0@0 | S_0@0 | S_0@0 |
| 72 | 75 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 | S_0@0 | S_0@0 |
| 73 | 75 | 100 |  | QPSK | 16 | P_16@0 | S_0@0 | S_0@0 | S_0@0 |
| 74 | 75 | 100 |  | QPSK | 16 | P_16@59 | S_0@0 | S_0@0 | S_0@0 |
| 75 | 75 | 100 |  | QPSK | 75 | P_75@0 | S_0@0 | S_0@0 | S_0@0 |
| 76 | 75 | 100 |  | 16QAM | 16 | P_16@0 | S_0@0 | S_0@0 | S_0@0 |
| 77 | 75 | 100 |  | 16QAM | 16 | P_16@59 | S_0@0 | S_0@0 | S_0@0 |
| 78 | 75 | 100 |  | 16QAM | 75 | P_75@0 | S_0@0 | S_0@0 | S_0@0 |
| 79 | 100 | 50 |  | QPSK | 18 | P_18@0 | S_0@0 | S_0@0 | S_0@0 |
| 80 | 100 | 50 |  | QPSK | 18 | P_18@82 | S_0@0 | S_0@0 | S_0@0 |
| 81 | 100 | 50 |  | QPSK | 100 | P_100@0 | S_0@0 | S_0@0 | S_0@0 |
| 82 | 100 | 50 |  | 16QAM | 18 | P_18@0 | S_0@0 | S_0@0 | S_0@0 |
| 83 | 100 | 50 |  | 16QAM | 18 | P_18@82 | S_0@0 | S_0@0 | S_0@0 |
| 84 | 100 | 50 |  | 16QAM | 100 | P_100@0 | S_0@0 | S_0@0 | S_0@0 |
| 85 | 100 | 75 |  | QPSK | 18 | P_18@0 | S_0@0 | S_0@0 | S_0@0 |
| 86 | 100 | 75 |  | QPSK | 18 | P_18@82 | S_0@0 | S_0@0 | S_0@0 |
| 87 | 100 | 75 |  | QPSK | 100 | P_100@0 | S_0@0 | S_0@0 | S_0@0 |
| 88 | 100 | 75 |  | 16QAM | 18 | P_18@0 | S_0@0 | S_0@0 | S_0@0 |
| 89 | 100 | 75 |  | 16QAM | 18 | P_18@82 | S_0@0 | S_0@0 | S_0@0 |
| 90 | 100 | 75 |  | 16QAM | 100 | P_100@0 | S_0@0 | S_0@0 | S_0@0 |
| 91 | 100 | 100 |  | QPSK | 18 | P_18@0 | S_0@0 | S_0@0 | S_0@0 |
| 92 | 100 | 100 |  | QPSK | 18 | P_18@82 | S_0@0 | S_0@0 | S_0@0 |
| 93 | 100 | 100 |  | QPSK | 100 | P_100@0 | S_0@0 | S_0@0 | S_0@0 |
| 94 | 100 | 100 |  | 16QAM | 18 | P_18@0 | S_0@0 | S_0@0 | S_0@0 |
| 95 | 100 | 100 |  | 16QAM | 18 | P_18@82 | S_0@0 | S_0@0 | S_0@0 |
| 96 | 100 | 100 |  | 16QAM | 100 | P_100@0 | S_0@0 | S_0@0 | S_0@0 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: The frequencies of PCC and SCCs shall be switched and tested in each configuration. |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A, Figure A.55 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.5.2A.1.5.4.1-1 to 6.5.2A.1.5.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.5.2A.1.5.4.3.

6.5.2A.1.5.4.2	Test procedure

1.	Configure SCCs according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCCs as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.5.2A.1.5.4.3.

3.	SS activates SCCs by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.5.2A.1.5.4.1-1 to 6.5.2A.1.5.4.1-2. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

6.	Measure the EVM and  on PCC using Global In-Channel Tx-Test (Annex E).

7.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is –36.8dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -36.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

8.	Measure the EVM and  on PCC using Global In-Channel Tx-Test (Annex E).

6.5.2A.1.5.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCCs configuration there are no additional message contents.

6.5.2A.1.5.5	Test requirement

The PUSCH EVM derived in E.4.2 shall not exceed 17,5 % for QPSK and BPSK, 12,5% for 16 QAM.

The PUSCH derived in E.4.6.2 shall not exceed 17,5 % when embedded with data symbols of QPSK and BPSK, 12,5% for 16 QAM.

#### 6.5.2A.2	Carrier leakage for CA

##### 6.5.2A.2.0	Minimum conformance requirements

For inter-band carrier aggregation, the relative carrier leakage power is a power ratio of the additive sinusoid waveform and the modulated waveform. The relative carrier leakage power shall not exceed the values specified in Table 6.5.2A.2.0-1.

For the intra-band non-contiguous carrier aggregation, the relative carrier leakage power is a power ratio of the additive sinusoid waveform and the modulated waveform. The relative carrier leakage power shall not exceed the values specified in Table 6.5.2A.2.0-1.

For the intra-band contiguous carrier aggregation, the relative carrier leakage power is a power ratio of the additive sinusoid waveform and the modulated waveform. The relative carrier leakage power shall not exceed the values specified in Table 6.5.2A.2.0-1.

Table 6.5.2A.2.0-1: Minimum requirements for Relative Carrier Leakage Power

| Carrier Leakage | Parameters | Relative Limit (dBc) |
| --- | --- | --- |
|  | Output power >0 dBm | -25 |
|  | -30 dBm ≤ Output power ≤0 dBm | -20 |
|  | -40 dBm Output power < -30 dBm | -10 |

The normative reference for this requirement is TS 36.101 [2] clause 6.5.2A.2.

##### 6.5.2A.2.1	Carrier leakage for CA (intra-band contiguous DL CA and UL CA)

6.5.2A.2.1.1	Test Purpose

Carrier leakage is an additive sinusoid waveform that is confined within the aggregated transmission bandwidth configuration. The carrier leakage requirement is defined for each component carrier and is measured on the component carrier with PRBs allocated. The measurement interval is one slot in the time domain.

6.5.2A.2.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 10 and forward that support intra-band contiguous DL CA and UL CA.

6.5.2A.2.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5.2A.2.0

6.5.2A.2.1.4	Test description

6.5.2A.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on E-UTRA CA configurations specified in table 5.4.2A.1-1. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 6.5.2A.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.2A.2.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause [4.3.1] for different CA bandwidth classes. PCC and SCC mapped onto component carriers CC according to the notation: PCC-SCC: CCi-CCj, which means PCC on CCi and SCC on CCj, with CCi/j frequencies defined in the corresponding intra-band contiguous CA band in TS36.508 [7] as above. |  |  |  |  | See Table 6.5.1A.1.4.1-1: PCC-SCC: CC1-CC2 and CC2-CC1 |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  | See Table 6.5.1A.1.4.1-1: |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |
|  | PCC NRB Note 1 | SCCs NRB Note 1 | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 25 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 |  |  |
| 2 | 25 | 25 |  | QPSK | 8 | P_8@17 | S_0@0 |  |  |
| 3 | 50 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 |  |  |
| 4 | 50 | 50 |  | QPSK | 12 | P_12@38 | S_0@0 |  |  |
| 5 | 50 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 |  |  |
| 6 | 50 | 25 |  | QPSK | 8 | P_8@42 | S_0@0 |  |  |
| 7 | 25 | 50 |  | QPSK | 8 | P_8@0 | S_0@0 |  |  |
| 8 | 25 | 50 |  | QPSK | 8 | P_8@17 | S_0@0 |  |  |
| 9 | 25 | 75 |  | QPSK | 8 | P_8@0 | S_0@0 |  |  |
| 10 | 25 | 75 |  | QPSK | 8 | P_8@17 | S_0@0 |  |  |
| 11 | 75 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 |  |  |
| 12 | 75 | 25 |  | QPSK | 8 | P_8@67 | S_0@0 |  |  |
| 13 | 75 | 75 | NA | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 14 | 75 | 75 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| 15 | 100 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 16 | 100 | 25 |  | QPSK | 8 | P_8@92 | S_0@0 | - | - |
| 17 | 25 | 100 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 18 | 25 | 100 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 19 | 100 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 20 | 100 | 50 |  | QPSK | 12 | P_12@88 | S_0@0 | - | - |
| 21 | 50 | 100 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 22 | 50 | 100 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 23 | 100 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 24 | 100 | 75 |  | QPSK | 16 | P_16@84 | S_0@0 | - | - |
| 25 | 75 | 100 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 26 | 75 | 100 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| 27 | 100 | 100 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| 28 | 100 | 100 |  | QPSK | 18 | P_18@82 | S_0@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508[7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to in Table 6.5.2A.2.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.5.2A.2.1.4.3.

6.5.2A.2.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.5.2A.2.1.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.5.2A.2.1.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is 3.2 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or 3.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

6.	Measure carrier leakage on PCC using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test.

7.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is -26.8 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -26.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

8.	Measure carrier leakage on PCC using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test.

9.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is -36.8dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -36.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

10.	Measure carrier leakage on PCC using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test.

6.5.2A.2.1.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

6.5.2A.2.1.5	Test requirement

Each of the 20 carrier leakage results, derived in Annex E.3.1, shall not exceed the values in table 6.5.2A.2.1.5-1.

Table 6.5.2A.2.1.5-1: Test requirements for Relative Carrier Leakage Power

| Carrier Leakage | Parameters | Relative Limit (dBc) |
| --- | --- | --- |
|  | f ≤ 3.0GHz: 3.2 dBm ±3.2dB 3.0GHz < f ≤ 4.2GHz: 3.5 dBm ±3.5dB | -24.2 |
|  | f ≤ 3.0GHz: -26.8 dBm ±3.2dB 3.0GHz < f ≤ 4.2GHz: -26.5 dBm ±3.5dB | -19.2 |
|  | f ≤ 3.0GHz: -36.8dBm±3.2dB 3.0GHz < f ≤ 4.2GHz: -36.5 dBm ±3.5dB | -9.2 |

##### 6.5.2A.2.2	Carrier leakage for CA (inter-band DL CA and UL CA)

6.5.2A.2.2.1	Test Purpose

Carrier leakage is an additive sinusoid waveform that is confined within the aggregated transmission bandwidth configuration. The carrier leakage requirement is defined for each component carrier and is measured on the component carrier with PRBs allocated. The measurement interval is one slot in the time domain.

6.5.2A.2.2.2	Test applicability

This test case applies to all types of E-UTRA UE Release 11 and forward that support inter-band DL CA and UL CA.

6.5.2A.2.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5.2A.2.0.

6.5.2A.2.2.4	Test description

6.5.2A.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on E-UTRA CA configurations specified in table 5.4.2A.1-2. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 6.5.2A.2.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.2A.2.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  | Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |
|  | PCC NRB Note 1 | SCCs NRB Note 1 | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 6 | 25 | NA | QPSK | 5 | P_5@0 | S_0@0 | - | - |
| 2 | 6 | 25 |  | QPSK | 1 | P_1@0 | S_0@0 | - | - |
| 3 | 6 | 25 |  | QPSK | 1 | P_1@5 | S_0@0 | - | - |
| 4 | 15 | 25 |  | QPSK | 4 | P_4@0 | S_0@0 | - | - |
| 5 | 15 | 25 |  | QPSK | 4 | P_4@11 | S_0@0 | - | - |
| 6 | 15 | 50 |  | QPSK | 4 | P_4@0 | S_0@0 | - | - |
| 7 | 15 | 50 |  | QPSK | 4 | P_4@11 | S_0@0 | - | - |
| 8 | 25 | 6 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 9 | 25 | 6 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 10 | 25 | 15 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 11 | 25 | 15 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 12 | 25 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 13 | 25 | 25 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 14 | 25 | 50 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 15 | 25 | 50 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 16 | 50 | 15 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 17 | 50 | 15 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 18 | 50 | 25 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 19 | 50 | 25 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 20 | 50 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 21 | 50 | 50 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 22 | 50 | 100 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 23 | 50 | 100 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 24 | 75 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 |  |  |
| 25 | 75 | 75 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| 26 | 100 | 50 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| 27 | 100 | 50 |  | QPSK | 18 | P_18@82 | S_0@0 | - | - |
| 28 | 100 | 100 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| 29 | 100 | 100 |  | QPSK | 18 | P_18@82 | S_0@0 | - | - |
| 30 | 25 | 75 |  | QPSK | 8 | P_8@0 | S_0@0 |  |  |
| 31 | 25 | 75 |  | QPSK | 8 | P_8@17 | S_0@0 |  |  |
| 32 | 75 | 25 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 33 | 75 | 25 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| 34 | 50 | 75 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 35 | 50 | 75 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 36 | 75 | 50 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 37 | 75 | 50 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| 38 | 75 | 100 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 39 | 75 | 100 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| 40 | 100 | 75 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| 41 | 100 | 75 |  | QPSK | 18 | P_18@82 | S_0@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: The frequencies of PCC and SCC shall be switched and tested in each configuration. |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL RMCs are set according to in Table 6.5.2A.2.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.5.2A.2.2.4.3.

6.5.2A.2.2.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.5.2A.3.2.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.5.2A.2.2.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is 3.2 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or 3.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

6.	Measure carrier leakage on PCC using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test.

7.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is -26.8 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -26.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

8.	Measure carrier leakage on PCC using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test.

9.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is -36.8dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -36.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

10.	Measure carrier leakage on PCC using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test.

6.5.2A.2.2.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

6.5.2A.2.2.5	Test requirement

Each of the 20 carrier leakage results, derived in Annex E.3.1, shall not exceed the values in table 6.5.2A.2.2.5-1.

Table 6.5.2A.2.2.5-1: Test requirements for Relative Carrier Leakage Power

| Carrier Leakage | Parameters | Relative Limit (dBc) |
| --- | --- | --- |
|  | f ≤ 3.0GHz: 3.2 dBm ±3.2dB 3.0GHz < f ≤ 4.2GHz: 3.5 dBm ±3.5dB | -24.2 |
|  | f ≤ 3.0GHz: -26.8 dBm ±3.2dB 3.0GHz < f ≤ 4.2GHz: -26.5 dBm ±3.5dB | -19.2 |
|  | f ≤ 3.0GHz: -36.8dBm±3.2dB 3.0GHz < f ≤ 4.2GHz: -36.5 dBm ±3.5dB | -9.2 |

##### 6.5.2A.2.3	Carrier leakage for CA (intra-band non-contiguous DL CA and UL CA)

6.5.2A.2.3.1	Test Purpose

Carrier leakage is an additive sinusoid waveform that is confined within the aggregated transmission bandwidth configuration. The carrier leakage requirement is defined for each component carrier and is measured on the component carrier with PRBs allocated. The measurement interval is one slot in the time domain.

6.5.2A.2.3.2	Test applicability

This test case applies to all types of E-UTRA UE Release 11 and forward that support intra-band non-contiguous DL CA and UL CA.

6.5.2A.2.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5.2A.2.0.

6.5.2A.2.3.4	Test description

6.5.2A.2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on E-UTRA CA configurations specified in table 5.4.2A.1-3. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 6.5.2A.2.3.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.2A.2.3.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Low Range for CC1; High Range for CC2 PCC-SCC: CC1-CC2 and CC2-CC1 |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  | Refer to test point Test only test points with Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  | Wgap [MHz] | DL Allocation |  | UL Allocation |  |  |  |
|  | PCC NRB | SCCs NRB |  | CC MOD | PCC & SCC RB allocation | CC MOD | PCC NRB_alloc (LCRB @ RBstart) | CC MOD | SCC NRB_alloc (LCRB @ RBstart) |
| Test Parameters for CA_4A-4A Configurations |  |  |  |  |  |  |  |  |  |
| 1 | 25 | 25 | 35 | QPSK | N/A | QPSK | P_8@0 | QPSK | S_0@0 |
| 2 | 25 | 25 | 35 | QPSK |  | QPSK | P_8@17 | QPSK | S_0@0 |
| 3 | 50 | 50 | 25 | QPSK |  | QPSK | P_12@0 | QPSK | S_0@0 |
| 4 | 50 | 50 | 25 | QPSK |  | QPSK | P_12@38 | QPSK | S_0@0 |
| 5 | 75 | 75 | 15 | QPSK |  | QPSK | P_16@0 | QPSK | S_0@0 |
| 6 | 75 | 75 | 15 | QPSK |  | QPSK | P_16@59 | QPSK | S_0@0 |
| 7 | 100 | 100 | 5 | QPSK |  | QPSK | P_18@0 | QPSK | S_0@0 |
| 8 | 100 | 100 | 5 | QPSK |  | QPSK | P_18@82 | QPSK | S_0@0 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-3. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL RMCs are set according to in Table 6.5.2A.2.3.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.5.2A.2.3.4.3.

6.5.2A.2.3.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.5.2A.3.2.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.5.2A.2.3.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is 3.2 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or 3.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

5.	Measure carrier leakage on PCC using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test.

6.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is -26.8 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -26.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

7.	Measure carrier leakage on PCC using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test.

8.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is -36.8dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -36.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

9.	Measure carrier leakage on PCC using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test.

6.5.2A.2.3.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

6.5.2A.2.3.5	Test requirement

Each of the 20 carrier leakage results, derived in Annex E.3.1, shall not exceed the values in table 6.5.2A.2.3.5-1.

Table 6.5.2A.2.3.5-1: Test requirements for Relative Carrier Leakage Power

| Carrier Leakage | Parameters | Relative Limit (dBc) |
| --- | --- | --- |
|  | f ≤ 3.0GHz: 3.2 dBm ±3.2dB 3.0GHz < f ≤ 4.2GHz: 3.5 dBm ±3.5dB | -24.2 |
|  | f ≤ 3.0GHz: -26.8 dBm ±3.2dB 3.0GHz < f ≤ 4.2GHz: -26.5 dBm ±3.5dB | -19.2 |
|  | f ≤ 3.0GHz: -36.8dBm±3.2dB 3.0GHz < f ≤ 4.2GHz: -36.5 dBm ±3.5dB | -9.2 |

##### 6.5.2A.2.4	Carrier leakage for CA (3UL CA)

6.5.2A.2.4.1	Test Purpose

Carrier leakage is an additive sinusoid waveform that is confined within the aggregated transmission bandwidth configuration. The carrier leakage requirement is defined for each component carrier and is measured on the component carrier with PRBs allocated. The measurement interval is one slot in the time domain.

6.5.2A.2.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 13 and forward that support intra-band contiguous or inter-band 3DL CA and 3UL CA.

6.5.2A.2.4.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5.2A.2.0.

6.5.2A.2.4.4	Test description

6.5.2A.2.4.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on E-UTRA CA configurations specified in table 5.4.2A.1-2 for CA configurations with two bands. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 6.5.2A.2.4.4.1-1 to 6.5.2A.2.4.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.2A.2.4.4.1-1: Test Configuration Table for intra-band contiguous CA

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Low range, High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  | Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |
|  | PCC NRB Note 1 | SCCs NRB Note 1 | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC1 & SCC2 RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 50 | 50 | N/A | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 | - |
| 2 | 50 | 50 |  | QPSK | 12 | P_12@38 | S_0@0 | S_0@0 | - |
| 3 | 50 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | - |
| 4 | 50 | 25 |  | QPSK | 8 | P_8@42 | S_0@0 | S_0@0 | - |
| 5 | 25 | 50 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | - |
| 6 | 25 | 50 |  | QPSK | 8 | P_8@17 | S_0@0 | S_0@0 | - |
| 7 | 75 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | S_0@0 | - |
| 8 | 75 | 75 |  | QPSK | 16 | P_16@59 | S_0@0 | S_0@0 | - |
| 9 | 100 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | - |
| 10 | 100 | 25 |  | QPSK | 8 | P_8@92 | S_0@0 | S_0@0 | - |
| 11 | 25 | 100 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | - |
| 12 | 25 | 100 |  | QPSK | 8 | P_8@17 | S_0@0 | S_0@0 | - |
| 13 | 100 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 | - |
| 14 | 100 | 50 |  | QPSK | 12 | P_12@88 | S_0@0 | S_0@0 | - |
| 15 | 50 | 100 |  | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 | - |
| 16 | 50 | 100 |  | QPSK | 12 | P_12@38 | S_0@0 | S_0@0 | - |
| 17 | 100 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | S_0@0 | - |
| 18 | 100 | 75 |  | QPSK | 16 | P_16@84 | S_0@0 | S_0@0 | - |
| 19 | 75 | 100 |  | QPSK | 16 | P_16@0 | S_0@0 | S_0@0 | - |
| 20 | 75 | 100 |  | QPSK | 16 | P_16@59 | S_0@0 | S_0@0 | - |
| 21 | 100 | 100 |  | QPSK | 18 | P_18@0 | S_0@0 | S_0@0 | - |
| 22 | 100 | 100 |  | QPSK | 18 | P_18@82 | S_0@0 | S_0@0 | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |  |

Table 6.5.2A.2.4.4.1-2: Test Configuration Table for inter-band CA

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  | Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |
|  | PCC NRB Note 1 | SCCs NRB Note 1 | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC1 & SCC2 RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 6 | 25 | N/A | QPSK | 5 | P_5@0 | S_0@0 | S_0@0 | - |
| 2 | 6 | 25 |  | QPSK | 1 | P_1@0 | S_0@0 | S_0@0 | - |
| 3 | 6 | 25 |  | QPSK | 1 | P_1@5 | S_0@0 | S_0@0 | - |
| 4 | 6 | 25 |  | QPSK | 4 | P_4@0 | S_0@0 | S_0@0 | - |
| 5 | 15 | 25 |  | QPSK | 4 | P_4@11 | S_0@0 | S_0@0 | - |
| 6 | 15 | 25 |  | QPSK | 4 | P_4@0 | S_0@0 | S_0@0 | - |
| 7 | 15 | 50 |  | QPSK | 4 | P_4@11 | S_0@0 | S_0@0 | - |
| 8 | 15 | 50 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | - |
| 9 | 25 | 6 |  | QPSK | 8 | P_8@17 | S_0@0 | S_0@0 | - |
| 10 | 25 | 6 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | - |
| 11 | 25 | 15 |  | QPSK | 8 | P_8@17 | S_0@0 | S_0@0 | - |
| 12 | 25 | 15 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | - |
| 13 | 25 | 25 |  | QPSK | 8 | P_8@17 | S_0@0 | S_0@0 | - |
| 14 | 25 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | - |
| 15 | 25 | 50 |  | QPSK | 8 | P_8@17 | S_0@0 | S_0@0 | - |
| 16 | 25 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 | - |
| 17 | 50 | 15 |  | QPSK | 12 | P_12@38 | S_0@0 | S_0@0 | - |
| 18 | 50 | 15 |  | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 | - |
| 19 | 50 | 25 |  | QPSK | 12 | P_12@38 | S_0@0 | S_0@0 | - |
| 20 | 50 | 25 |  | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 | - |
| 21 | 50 | 50 |  | QPSK | 12 | P_12@38 | S_0@0 | S_0@0 | - |
| 22 | 50 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 | - |
| 23 | 50 | 100 |  | QPSK | 12 | P_12@38 | S_0@0 | S_0@0 | - |
| 24 | 50 | 100 |  | QPSK | 16 | P_16@0 | S_0@0 | S_0@0 | - |
| 25 | 75 | 75 |  | QPSK | 16 | P_16@59 | S_0@0 | S_0@0 | - |
| 26 | 75 | 75 |  | QPSK | 18 | P_18@0 | S_0@0 | S_0@0 | - |
| 27 | 100 | 50 |  | QPSK | 18 | P_18@82 | S_0@0 | S_0@0 | - |
| 28 | 100 | 50 |  | QPSK | 18 | P_18@0 | S_0@0 | S_0@0 | - |
| 29 | 100 | 100 |  | QPSK | 18 | P_18@82 | S_0@0 | S_0@0 | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: The frequencies of PCC and SCC shall be switched and tested in each configuration. |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A, Figure A.55 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.5.2A.2.4.4.1-1 to 6.5.2A.2.4.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.5.2A.2.4.4.3.

6.5.2A.2.4.4.2	Test procedure

1.	Configure SCCs according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCCs as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.5.2A.2.4.4.3.

3.	SS activates SCCs by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.5.2A.2.4.4.1-1 to 6.5.2A.2.4.4.1-2. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is 3.2 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or 3.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

6.	Measure carrier leakage on PCC using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test.

7.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is -26.8 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -26.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

8.	Measure carrier leakage on PCC using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test.

9.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is -36.8dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -36.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

10.	Measure carrier leakage on PCC using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test.

6.5.2A.2.4.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCCs configuration there are no additional message contents.

6.5.2A.2.4.5	Test requirement

Each of the 20 carrier leakage results, derived in Annex E.3.1, shall not exceed the values in table 6.5.2A.2.3.5-1.

Table 6.5.2A.2.4.5-1: Test requirements for Relative Carrier Leakage Power

| Carrier Leakage | Parameters | Relative Limit (dBc) |
| --- | --- | --- |
|  | f ≤ 3.0GHz: 3.2 dBm ±3.2dB 3.0GHz < f ≤ 4.2GHz: 3.5 dBm ±3.5dB | -24.2 |
|  | f ≤ 3.0GHz: -26.8 dBm ±3.2dB 3.0GHz < f ≤ 4.2GHz: -26.5 dBm ±3.5dB | -19.2 |
|  | f ≤ 3.0GHz: -36.8dBm±3.2dB 3.0GHz < f ≤ 4.2GHz: -36.5 dBm ±3.5dB | -9.2 |

##### 6.5.2A.2.5	Carrier leakage for CA (4UL CA)

6.5.2A.2.5.1	Test Purpose

Carrier leakage is an additive sinusoid waveform that is confined within the aggregated transmission bandwidth configuration. The carrier leakage requirement is defined for each component carrier and is measured on the component carrier with PRBs allocated. The measurement interval is one slot in the time domain.

6.5.2A.2.5.2	Test applicability

This test case applies to all types of E-UTRA UE release 11 and forward that support 4DL CA and 4UL CA.

6.5.2A.2.5.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5.2A.2.0.

6.5.2A.2.5.4	Test description

6.5.2A.2.5.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on E-UTRA CA configurations specified in table 5.4.2A.1. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 6.5.2A.2.5.4.1-1 to 6.5.2A.2.5.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.2A.2.5.4.1-1: Test Configuration Table for intra-band contiguous CA

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Low range, High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  | Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |
|  | PCC NRB Note 1 | SCCs NRB Note 1 | PCC & SCCs RB allocation |  | NRB_alloc | PCC & SCC1 & SCC2 & SCC3 RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 50 | 50 | N/A | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 | S_0@0 |
| 2 | 50 | 50 |  | QPSK | 12 | P_12@38 | S_0@0 | S_0@0 | S_0@0 |
| 3 | 50 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | S_0@0 |
| 4 | 50 | 25 |  | QPSK | 8 | P_8@42 | S_0@0 | S_0@0 | S_0@0 |
| 5 | 25 | 50 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | S_0@0 |
| 6 | 25 | 50 |  | QPSK | 8 | P_8@17 | S_0@0 | S_0@0 | S_0@0 |
| 7 | 75 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | S_0@0 | S_0@0 |
| 8 | 75 | 75 |  | QPSK | 16 | P_16@59 | S_0@0 | S_0@0 | S_0@0 |
| 9 | 100 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | S_0@0 |
| 10 | 100 | 25 |  | QPSK | 8 | P_8@92 | S_0@0 | S_0@0 | S_0@0 |
| 11 | 25 | 100 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | S_0@0 |
| 12 | 25 | 100 |  | QPSK | 8 | P_8@17 | S_0@0 | S_0@0 | S_0@0 |
| 13 | 100 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 | S_0@0 |
| 14 | 100 | 50 |  | QPSK | 12 | P_12@88 | S_0@0 | S_0@0 | S_0@0 |
| 15 | 50 | 100 |  | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 | S_0@0 |
| 16 | 50 | 100 |  | QPSK | 12 | P_12@38 | S_0@0 | S_0@0 | S_0@0 |
| 17 | 100 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | S_0@0 | S_0@0 |
| 18 | 100 | 75 |  | QPSK | 16 | P_16@84 | S_0@0 | S_0@0 | S_0@0 |
| 19 | 75 | 100 |  | QPSK | 16 | P_16@0 | S_0@0 | S_0@0 | S_0@0 |
| 20 | 75 | 100 |  | QPSK | 16 | P_16@59 | S_0@0 | S_0@0 | S_0@0 |
| 21 | 100 | 100 |  | QPSK | 18 | P_18@0 | S_0@0 | S_0@0 | S_0@0 |
| 22 | 100 | 100 |  | QPSK | 18 | P_18@82 | S_0@0 | S_0@0 | S_0@0 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |  |

Table 6.5.2A.2.5.4.1-2: Test Configuration Table for inter-band CA

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  | Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |
|  | PCC NRB Note 1 | SCCs NRB Note 1 | PCC & SCCs RB allocation |  | NRB_alloc | PCC & SCC1 & SCC2 & SCC3 RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 6 | 25 | N/A | QPSK | 5 | P_5@0 | S_0@0 | S_0@0 | S_0@0 |
| 2 | 6 | 25 |  | QPSK | 1 | P_1@0 | S_0@0 | S_0@0 | S_0@0 |
| 3 | 6 | 25 |  | QPSK | 1 | P_1@5 | S_0@0 | S_0@0 | S_0@0 |
| 4 | 6 | 25 |  | QPSK | 4 | P_4@0 | S_0@0 | S_0@0 | S_0@0 |
| 5 | 15 | 25 |  | QPSK | 4 | P_4@11 | S_0@0 | S_0@0 | S_0@0 |
| 6 | 15 | 25 |  | QPSK | 4 | P_4@0 | S_0@0 | S_0@0 | S_0@0 |
| 7 | 15 | 50 |  | QPSK | 4 | P_4@11 | S_0@0 | S_0@0 | S_0@0 |
| 8 | 15 | 50 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | S_0@0 |
| 9 | 25 | 6 |  | QPSK | 8 | P_8@17 | S_0@0 | S_0@0 | S_0@0 |
| 10 | 25 | 6 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | S_0@0 |
| 11 | 25 | 15 |  | QPSK | 8 | P_8@17 | S_0@0 | S_0@0 | S_0@0 |
| 12 | 25 | 15 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | S_0@0 |
| 13 | 25 | 25 |  | QPSK | 8 | P_8@17 | S_0@0 | S_0@0 | S_0@0 |
| 14 | 25 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | S_0@0 |
| 15 | 25 | 50 |  | QPSK | 8 | P_8@17 | S_0@0 | S_0@0 | S_0@0 |
| 16 | 25 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 | S_0@0 |
| 17 | 50 | 15 |  | QPSK | 12 | P_12@38 | S_0@0 | S_0@0 | S_0@0 |
| 18 | 50 | 15 |  | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 | S_0@0 |
| 19 | 50 | 25 |  | QPSK | 12 | P_12@38 | S_0@0 | S_0@0 | S_0@0 |
| 20 | 50 | 25 |  | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 | S_0@0 |
| 21 | 50 | 50 |  | QPSK | 12 | P_12@38 | S_0@0 | S_0@0 | S_0@0 |
| 22 | 50 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 | S_0@0 |
| 23 | 50 | 100 |  | QPSK | 12 | P_12@38 | S_0@0 | S_0@0 | S_0@0 |
| 24 | 50 | 100 |  | QPSK | 16 | P_16@0 | S_0@0 | S_0@0 | S_0@0 |
| 25 | 75 | 75 |  | QPSK | 16 | P_16@59 | S_0@0 | S_0@0 | S_0@0 |
| 26 | 75 | 75 |  | QPSK | 18 | P_18@0 | S_0@0 | S_0@0 | S_0@0 |
| 27 | 100 | 50 |  | QPSK | 18 | P_18@82 | S_0@0 | S_0@0 | S_0@0 |
| 28 | 100 | 50 |  | QPSK | 18 | P_18@0 | S_0@0 | S_0@0 | S_0@0 |
| 29 | 100 | 100 |  | QPSK | 18 | P_18@82 | S_0@0 | S_0@0 | S_0@0 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: The frequencies of PCC and SCC shall be switched and tested in each configuration. |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A, Figure A.55 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.5.2A.2.5.4.1-1 to 6.5.2A.2.5.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.5.2A.2.5.4.3.

6.5.2A.2.5.4.2	Test procedure

1.	Configure SCCs according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCCs as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.5.2A.2.5.4.3.

3.	SS activates SCCs by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.5.2A.2.5.4.1-1 to 6.5.2A.2.5.4.1-2. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is 3.2 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or 3.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

6.	Measure carrier leakage on PCC using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test.

7.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is -26.8 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -26.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

8.	Measure carrier leakage on PCC using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test.

9.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is -36.8dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -36.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

10.	Measure carrier leakage on PCC using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test.

6.5.2A.2.5.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCCs configuration there are no additional message contents.

6.5.2A.2.5.5	Test requirement

Each of the 20 carrier leakage results, derived in Annex E.3.1, shall not exceed the values in table 6.5.2A.2.3.5-1.

Table 6.5.2A.2.5.5-1: Test requirements for Relative Carrier Leakage Power

| Carrier Leakage | Parameters | Relative Limit (dBc) |
| --- | --- | --- |
|  | f ≤ 3.0GHz: 3.2 dBm ±3.2dB 3.0GHz < f ≤ 4.2GHz: 3.5 dBm ±3.5dB | -24.2 |
|  | f ≤ 3.0GHz: -26.8 dBm ±3.2dB 3.0GHz < f ≤ 4.2GHz: -26.5 dBm ±3.5dB | -19.2 |
|  | f ≤ 3.0GHz: -36.8dBm±3.2dB 3.0GHz < f ≤ 4.2GHz: -36.5 dBm ±3.5dB | -9.2 |

#### 6.5.2A.3	In-band emissions for non allocated RB for CA

##### 6.5.2A.3.0	Minimum conformance requirements

For inter-band carrier aggregation with uplink assigned to two E-UTRA bands, the requirements shall apply on each component carrier as defined in clause 6.5.2 with all component carriers active. If two contiguous component carriers are assigned to one E-UTRA band, the requirements in subclauses 6.5.2A.1, 6.5.2A.2, and 6.5.2A.3 apply for those component carriers.

The requirements in this clause apply with PCC and SCC in the UL configured and activated: PCC with PRB allocation and SCC without PRB allocation and without CSI reporting and SRS configured.

For intra-band contiguous carrier aggregation bandwidth class B and C, the requirements in Table 6.5.2A.3.0-1 and 6.5.2A.3.0-2 apply within the aggregated transmission bandwidth configuration with both component carrier (s) active and one single contiguous PRB allocation of bandwidth at the edge of the aggregated transmission bandwidth configuration.

The inband emission is defined as the interference falling into the non allocated resource blocks for all component carriers. The measurement method for the inband emissions in the component carrier with PRB allocation is specified in annex F. For a non allocated component carrier a spectral measurement is specified.

For intra-band non-contiguous carrier aggregation the requirements for in-band emissions should be defined for each component carrier. Requirements only apply with PRB allocation in one of the component carriers according to Table 6.5.2.3.3.

Table 6.5.2A.3.0-1: Minimum requirements for in-band emissions (allocated component carrier)

| Parameter | Unit | Limit |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB |  |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -25 |  | Exception for IQ image (NOTE 3) |
| Carrier leakage | dBc | -25 | Output power > 0 dBm | Exception for Carrier frequency (NOTE 4) |
|  |  | -20 | -30 dBm ≤ Output power ≤ 0 dBm |  |
|  |  | -10 | -40 dBm Output power < -30 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB - 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in NOTE 9. The limit is evaluated in each non-allocated RB. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs NOTE 3: Exceptions to the general limit are allowed for up to +1 RBs within a contiguous width of +1 non-allocated RBs. The measurement bandwidth is 1 RB. NOTE 4: Exceptions to the general limit are allowed for up to two contiguous non-allocated RBs. The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in the non-allocated RB to the measured total power in all allocated RBs. NOTE 5: is the Transmission Bandwidth (see Figure 5.4.2-1) not exceeding NOTE 6: is the Transmission Bandwidth Configuration (see Figure 5.4.2-1) of the component carrier with RBs allocated. NOTE 7: is the limit specified in Table 6.5.2.1.3-1 for the modulation format used in the allocated RBs. NOTE 8: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. or for the first adjacent RB outside of the allocated bandwidth). NOTE 9: is the transmitted power per 180 kHz in allocated RBs, measured in dBm. |  |  |  |  |

Table 6.5.2A.3.0-2: Minimum requirements for in-band emissions (not allocated component carrier)

| Para-meter | Unit | Meas BW NOTE 1 | Limit |  | remark | Applicable Frequencies |
| --- | --- | --- | --- | --- | --- | --- |
| General | dB | BW of 1 RB (180KHz rectangular) |  |  | The reference value is the average power per allocated RB in the allocated component carrier | Any RB in the non allocated component carrier. The frequency raster of the RBs is derived when this component carrier is allocated with RBs |
| IQ Image | dB | BW of 1 RB (180KHz rectangular) | -25 (NOTE 2) |  | The reference value is the average power per allocated RB in the allocated component carrier | The frequencies of the contiguous non-allocated RBs are unknown. The frequency raster of the RBs is derived when this component carrier is allocated with RBs |
| Carrier leakage | dBc | BW of 1 RB (180KHz rectangular) | NOTE 3 |  | The reference value is the total power of the allocated RBs in the allocated component carrier | The frequencies of the up to 2 non-allocated RBs are unknown. The frequency raster of the RBs is derived when this component carrier is allocated with RBs |
|  |  |  | -25 | Output power > 0 dBm |  |  |
|  |  |  | -20 | -30 dBm ≤ Output power ≤ 0 dBm |  |  |
|  |  |  | -10 | -40 dBm Output power < -30 dBm |  |  |
| NOTE1: Resolution BWs smaller than the measurement BW may be integrated to achieve the measurement bandwidth. NOTE 2: Exceptions to the general limit are allowed for up to +1 RBs within a contiguous width of +1 non-allocated RBs. NOTE 3: Two Exceptions to the general limit are allowed for up to two contiguous non-allocated RBs NOTE 4: NOTES 1, 5, 6, 7, 8, 9 from Table 6.5.2A.3.0-1 apply for Table 6.5.2A.3.0-2 as well. NOTE 5: for measured non-allocated RB in the non allocated component carrier may take non-integer values when the carrier spacing between the CCs is not a multiple of RB. |  |  |  |  |  |  |

The normative reference for this requirement is TS 36.101[2] clause 6.5.2A.3.

##### 6.5.2A.3.1	In-band emissions for non allocated RB for CA (intra-band contiguous DL CA and UL CA)

6.5.2A.3.1.1	Test Purpose

The in-band emissions are a measure of the interference falling into the non-allocated resources blocks.

For an allocated component carrier, the in-band emission is defined as the average across 12 sub-carrier and as a function of the RB offset from the edge of the allocated UL transmission bandwidth. The in-band emission is measured as the ratio of the UE output power in a non–allocated RB to the UE output power in an allocated RB. The basic in-band emissions measurement interval is defined over one slot in the time domain. When the PUSCH or PUCCH transmission slot is shortened due to multiplexing with SRS, the in-band emissions measurement interval is reduced by one SC-FDMA symbol, accordingly.

For a non allocated component carrier a spectral measurement is specified.

6.5.2A.3.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 10 and forward that support intra-band contiguous DL CA and UL CA.

6.5.2A.3.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5.2A.3.0.

6.5.2A.3.1.4	Test description

6.5.2A.3.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on E-UTRA CA configurations specified in table 5.4.2A.1-1. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 6.5.2A.3.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.2A.3.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. PCC and SCC mapped onto component carriers CC according to the notation: PCC-SCC: CCi-CCj, which means PCC on CCi and SCC on CCj, with CCi/j frequencies defined in the corresponding intra-band contiguous CA band in TS36.508 [7] as above. |  |  |  |  | See Table 6.5.1A.1.4.1-1: PCC-SCC: CC1-CC2, CC2-CC1 |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  | See Table 6.5.1A.1.4.1-1: |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |
|  | PCC NRB | SCC NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 Note 2 | 25 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 2 Note 3 | 25 | 25 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 3 Note 2 | 25 | 75 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 4 Note 3 | 25 | 75 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 5 Note 2 | 75 | 25 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 6 Note 3 | 75 | 25 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| 7 Note 2 | 75 | 75 | N.A. | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 8 Note 3 | 75 | 75 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| 9 Note 2 | 100 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 10 Note 3 | 100 | 25 |  | QPSK | 8 | P_8@92 | S_0@0 | - | - |
| 11 Note 2 | 25 | 100 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 12 Note 3 | 25 | 100 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 13 Note 2 | 100 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 14 Note 3 | 100 | 50 |  | QPSK | 12 | P_12@88 | S_0@0 | - | - |
| 15 Note 2 | 50 | 100 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 16 Note 3 | 50 | 100 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 17 Note 2 | 100 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 18 Note 3 | 100 | 75 |  | QPSK | 16 | P_16@84 | S_0@0 | - | - |
| 19 Note 2 | 75 | 100 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 20 Note 3 | 75 | 100 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| 21 Note 2 | 100 | 100 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| 22 Note 3 | 100 | 100 |  | QPSK | 18 | P_18@82 | S_0@0 | - | - |
| 23 Note 2 | 50 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 24 Note 3 | 50 | 50 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 25 Note 2 | 25 | 50 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 26 Note 3 | 25 | 50 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 27 Note 2 | 50 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 28 Note 3 | 50 | 25 |  | QPSK | 8 | P_8@42 | S_0@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: Applicable only for PCC-SCC: CC1-CC2. Note 3: Applicable only for PCC-SCC: CC2-CC1. Note 4: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508[7] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to in Table 6.5.2A.3.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.5.2A.3.1.4.3.

6.5.2A.3.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.5.2A.3.1.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.5.2A.3.1.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is 3.2 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or 3.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

6.	Measure In-band emission on PCC using Global In-Channel Tx-Test (Annex E). Measure power spectral density on the SCC. For TDD slots with transient periods are not under test.

7.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is -26.8 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -26.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

8.	Measure In-band emission on PCC using Global In-Channel Tx-Test (Annex E). Measure power spectral density on the SCC. For TDD slots with transient periods are not under test.

9.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is to -36.8 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -36.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

10.	Measure In-band emission on PCC using Global In-Channel Tx-Test (Annex E). Measure power spectral density on the SCC. For TDD slots with transient periods are not under test.

6.5.2A.3.1.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

6.5.2A.3.1.5	Test requirement

Each of the 20 In-band emissions results, derived in Annex E.4.3 shall not exceed the corresponding values in Table 6.5.2A.3.1.5-1.

Table 6.5.2A.3.1.5-1: Test requirements for in-band emissions (allocated component carrier)

| Parameter Description | Unit | Limit (Note 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | +0.8 |  | Any non-allocated (Note 2) |
| IQ Image | dB | -24.2 |  | Image frequencies (Notes 2, 3) |
| Carrier leakage | dBc | -24.2 | Output power f ≤ 3.0GHz: 3.2dBm ±3.2dB 3.0GHz < f ≤ 4.2GHz: 3.5 dBm ±3.5dB | LO frequency (Notes 4, 5) |
|  |  | -19.2 | Output power f ≤ 3.0GHz: -26.8 dBm ±3.2dB 3.0GHz < f ≤ 4.2GHz: -26.5 dBm ±3.5dB |  |
|  |  | -9.2 | Output power f ≤ 3.0GHz: -36.8 dBm ±3.2dB 3.0GHz < f ≤ 4.2GHz: -36.5 dBm ±3.5dB |  |
| Note 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the test requirement is calculated as the higher of PRB – 29.2 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in Note 10. Note 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. Note 3: The applicable frequencies for this limit are not known due to unknown LO frequency. See Note 3 in Table 6.5.2A.3.0-1. Note 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. Note 5: The applicable frequencies for this limit are not known due to unknown LO frequency. See Note 4 in Table 6.5.2A.3.0-1. Note 6: is the Transmission Bandwidth (see Figure 5.4.2-1). Note 7: is the Transmission Bandwidth Configuration (see Figure 5.4.2-1) of the component carrier with RBs allocated. Note 8: is the limit specified in Table 6.5.2.1.3-1 for the modulation format used in the allocated RBs. Note 9: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. or for the first adjacent RB outside of the allocated bandwidth). Note 10: is the transmitted power per 180 kHz in allocated RBs, measured in dBm. |  |  |  |  |

The in-band emissions results, measured with the spectral test shall not exceed the corresponding values in Table 6.5.2A.3.1.5-2

Table 6.5.2A.3.1.5-2: Test requirements for in-band emissions (not allocated component carrier)

| Para-meter | Unit | Meas BW Note 1 | Limit |  | remark | Applicable Frequencies |
| --- | --- | --- | --- | --- | --- | --- |
| General | dB | BW of 1 RB (180KHz rectangular) |  |  | The reference value is the average power per allocated RB in the allocated component carrier | Any RB in the non allocated component carrier. The frequency raster of the RBs is derived when this component carrier is allocated with RBs |
| IQ Image | dB | BW of 1 RB (180KHz rectangular) | -24.2 Note 2 |  | The reference value is the average power per allocated RB in the allocated component carrier | The frequencies of thecontiguous non-allocated RBs are unknown. The frequency raster of the RBs is derived when this component carrier is allocated with RBs |
| Carrier leakage | dBc | BW of 1 RB (180KHz rectangular) | Note 3 |  | The reference value is the total power of the allocated RBs in the allocated component carrier | The frequencies of the up to 2 non-allocated RBs are unknown. The frequency raster of the RBs is derived when this component carrier is allocated with RBs |
|  |  |  | -24.2 | Output power > 0 dBm |  |  |
|  |  |  | -19.2 | -30 dBm ≤ Output power ≤ 0 dBm |  |  |
|  |  |  | -9.2 | -40 dBm Output power < -30 dBm |  |  |
| Note1: Resolution BWs smaller than the measurement BW may be integrated to achieve the measurement bandwidth. Note 2: Exceptions to the general limit are allowed for up to +1 RBs within a contiguous width of +1 non-allocated RBs. Note 3: Exceptions to the general limit are allowed for up to two contiguous non-allocated RBs. Note 4: Note s1, 6, 7, 8, 9, 10 from Table 6.5.2A.3.0-1 apply for Table 6.5.2A.3.0-2 as well. Note 5: for measured non-allocated RB in the non allocated component carrier may take non-integer values when the carrier spacing between the CCs is not a multiple of RB. |  |  |  |  |  |  |

##### 6.5.2A.3.2	In-band emissions for non allocated RB for CA (inter-band DL CA and UL CA)

6.5.2A.3.2.1	Test Purpose

Same test purpose as in clause 6.5.2A.3.1.1.

6.5.2A.3.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 11 and forward that support inter-band DL CA and UL CA.

This test case also applies to all types of E-UTRA UE release 14 and forward that supports uplink LAA.

6.5.2A.3.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5.2A.3.0.

6.5.2A.3.2.4	Test description

6.5.2A.3.2.4.1	Initial conditions

Same as in clause 6.5.2A.3.1.4.1 with the following exceptions:

-	Instead of Table 5.4.2A.1-1 use Table 5.4.2A.1-2 for CA configurations with two bands or Table 5.4.2A.1-2a for CA configurations with three bands.

-	Instead of Table 6.5.2A.3.1.4.1-1  use Table 6.5.2A.3.2.4.1-1.

-	Instead of clause 6.5.2A.3.1.4.3  use clause 6.5.2A.3.2.4.3.

Table 6.5.2A.3.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 6 | 25 | N/A for this test | QPSK | 1 | P_1@0 | S_0@0 | - | - |
| 6 | 25 |  | QPSK | 1 | P_1@5 | S_0@0 | - | - |
| 6 | 50 |  | QPSK | 1 | P_1@0 | S_0@0 | - | - |
| 6 | 50 |  | QPSK | 1 | P_1@5 | S_0@0 | - | - |
| 15 | 25 |  | QPSK | 4 | P_4@0 | S_0@0 | - | - |
| 15 | 25 |  | QPSK | 4 | P_4@11 | S_0@0 | - | - |
| 25 | 6 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 25 | 6 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 25 | 15 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 25 | 15 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 25 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 25 | 25 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 25 | 50 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 25 | 50 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 50 | 6 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 50 | 6 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 50 | 25 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 50 | 25 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 50 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 50 | 50 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 50 | 100 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 50 | 100 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 75 | 25 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 75 | 25 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| 75 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 75 | 75 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| 75 | 100 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 75 | 100 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| 100 | 50 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| 100 | 50 |  | QPSK | 18 | P_18@82 | S_0@0 | - | - |
| 100 | 75 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| 100 | 75 |  | QPSK | 18 | P_18@82 | S_0@0 | - | - |
| 100 | 100 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| 100 | 100 |  | QPSK | 18 | P_18@82 | S_0@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. Note 3: The frequencies of PCC and SCC shall be switched and tested in each configuration. |  |  |  |  |  |  |  |  |

Table 6.5.2A.3.2.4.1-2: Test Configuration Table for Uplink LAA CA

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 1) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| Config ID | PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC RB Allocation (LCRB @ RBstart) SCC RB Allocation(L@ RBstart) Note2 |  |  |  |
| 1 | 25 | 100 | N/A for this test | QPSK | 10 | P_0@0 | S_1@1 |  |  |
| 2 | 25 | 100 |  | 16QAM | 10 | P_0@0 | S_1@1 |  |  |
| 3 | 25 | 100 |  | 64QAM | 10 | P_0@0 | S_1@1 |  |  |
| 4 | 50 | 100 |  | QPSK | 10 | P_0@0 | S_1@1 |  |  |
| 5 | 50 | 100 |  | 16QAM | 10 | P_0@0 | S_1@1 |  |  |
| 6 | 50 | 100 |  | 64QAM | 10 | P_0@0 | S_1@1 |  |  |
| 7 | 75 | 100 |  | QPSK | 10 | P_0@0 | S_1@1 |  |  |
| 8 | 75 | 100 |  | 16QAM | 10 | P_0@0 | S_1@1 |  |  |
| 9 | 75 | 100 |  | 64QAM | 10 | P_0@0 | S_1@1 |  |  |
| 10 | 100 | 100 |  | QPSK | 10 | P_0@0 | S_1@1 |  |  |
| 11 | 100 | 100 |  | 16QAM | 10 | P_0@0 | S_1@1 |  |  |
| 12 | 100 | 100 |  | 64QAM | 10 | P_0@0 | S_1@1 |  |  |
| 13 | 25 | 100 |  | QPSK | 10 | P_0@0 | S_1@5 |  |  |
| 14 | 25 | 100 |  | 16QAM | 10 | P_0@0 | S_1@5 |  |  |
| 15 | 25 | 100 |  | 64QAM | 10 | P_0@0 | S_1@5 |  |  |
| 16 | 50 | 100 |  | QPSK | 10 | P_0@0 | S_1@5 |  |  |
| 17 | 50 | 100 |  | 16QAM | 10 | P_0@0 | S_1@5 |  |  |
| 18 | 50 | 100 |  | 64QAM | 10 | P_0@0 | S_1@5 |  |  |
| 19 | 75 | 100 |  | QPSK | 10 | P_0@0 | S_1@5 |  |  |
| 20 | 75 | 100 |  | 16QAM | 10 | P_0@0 | S_1@5 |  |  |
| 21 | 75 | 100 |  | 64QAM | 10 | P_0@0 | S_1@5 |  |  |
| 22 | 100 | 100 |  | QPSK | 10 | P_0@0 | S_1@5 |  |  |
| 23 | 100 | 100 |  | 16QAM | 10 | P_0@0 | S_1@5 |  |  |
| 24 | 100 | 100 |  | 64QAM | 10 | P_0@0 | S_1@5 |  |  |
|  | Note 1: CA Configuration, Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths for uplink LAA are specified in Table 5.4.2A.1-2. Note 2: For eLAA SCC, the RBstart and L are defined in TS 36.213[10] clause 8.1.4. |  |  |  |  |  |  |  |  |

6.5.2A.3.2.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.5.2A.3.2.4.3.

3	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 (or DCI format 0A for SCC in the case of uplink LAA) for C_RNTI to schedule the UL RMC according to Table 6.5.2A.3.2.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is 13.2 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or 13.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

6.	Measure In-band emission on PCC using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test.

7.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is 3.2 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or 3.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

8.	Measure In-band emission on PCC using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test.

9.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is -26.8 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -26.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

10.	Measure In-band emission on PCC using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test.

11.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is to -36.8 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -36.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

12.	Measure In-band emission on PCC using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test.

6.5.2A.3.2.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

For SCC in the case of uplink LAA, message contents are according to TS 36.508 [7] subclause4.6 with the exceptions specified in Tables 6.5.2A.1.2.4.3-1 and 6.5.2A.1.2.4.3-2.

6.5.2A.3.2.5	Test requirement

Each of the 20 In-band emissions results, derived in Annex E.4.3 shall not exceed the corresponding values in Table 6.5.2A.3.2.5-1.

Table 6.5.2A.3.2.5-1: Test requirements for in-band emissions

| Parameter description | Unit | Limit (Note 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | +0.8 |  | Any non-allocated (Note 2) |
| IQ Image | dB | -27.2 | Image frequencies when carrier centre frequency < 1 GHz and Output power > 10 dBm | Image frequencies (Notes 2, 3) |
|  |  | -24.2 | Image frequencies when carrier centre frequency < 1 GHz and Output power ≤ 10 dBm |  |
|  |  | -24.2 | Image frequencies when carrier centre frequency ≥ 1 GHz |  |
| Carrier leakage | dBc | -27.2 | Output power > 10 dBm and carrier centre frequency < 1 GHz | Carrier frequency (Notes 4, 5) |
|  |  | -24.2 | Output power > 10 dBm and carrier centre frequency ≥ 1 GHz |  |
|  |  | -24.2 | 0 dBm ≤ Output power ≤10 dBm |  |
|  |  | -19.2 | -30 dBm ≤ Output power ≤ 0 dBm |  |
|  |  | -9.2 | -40 dBm Output power < -30 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB - 29.2 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in Note 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the centre carrier frequency, but excluding any allocated RBs. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit are those that are enclosed in the RBs containing the DC frequency if is odd, or in the two RBs immediately adjacent to the DC frequency if is even, but excluding any allocated RB. NOTE 6: is the Transmission Bandwidth (see Figure 5.4.2-1). NOTE 7: is the Transmission Bandwidth Configuration (see Figure 5.4.2-1). NOTE 8: is the limit specified in Table 6.5.2.1.3-1 for the modulation format used in the allocated RBs. NOTE 9: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. or for the first adjacent RB outside of the allocated bandwidth. NOTE 10: is the transmitted power per 180 kHz in allocated RBs, measured in dBm. |  |  |  |  |

##### 6.5.2A.3.3	In-band emissions for non allocated RB for CA (intra-band non-contiguous DL CA and UL CA)

6.5.2A.3.3.1	Test Purpose

Same test purpose as in clause 6.5.2A.3.1.1.

6.5.2A.3.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 11 and forward that support intra-band non-contiguous DL CA and UL CA.

6.5.2A.3.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5.2A.3.0.

6.5.2A.3.3.4	Test description

6.5.2A.3.3.4.1	Initial conditions

Same as in clause 6.5.2A.3.1.4.1 with the following exceptions:

-	Instead of Table 5.4.2A.1-1 use Table 5.4.2A.1-3 for CA configurations.

-	Instead of Table 6.5.2A.3.1.4.1-1  use Table 6.5.2A.3.3.4.1-1.

-	Instead of clause 6.5.2A.3.1.4.3  use clause 6.5.2A.3.3.4.3.

Table 6.5.2A.3.3.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Refer to test points A: N/A (Range is not relevant), Maximum Wgap |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  | Refer to test point Test only test points with Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  | Wgap [MHz] | DL Allocation |  | UL Allocation |  |  |  |
|  | PCC NRB | SCCs NRB |  | CC MOD | PCC & SCC RB allocation | CC MOD | PCC NRB_alloc (LCRB @ RBstart) | CC MOD | SCC NRB_alloc (LCRB @ RBstart) |
| Test Parameters for CA_4A-4A Configurations |  |  |  |  |  |  |  |  |  |
| 1 | 25 | 25 | 35 | N/A |  | QPSK | P_8@0 | QPSK | S_0@0 |
| 2 | 25 | 25 | 35 |  |  | QPSK | P_8@17 | QPSK | S_0@0 |
| 3 | 50 | 50 | 25 |  |  | QPSK | P_12@0 | QPSK | S_0@0 |
| 4 | 50 | 50 | 25 |  |  | QPSK | P_12@38 | QPSK | S_0@0 |
| 5 | 100 | 100 | 5 |  |  | QPSK | P_18@0 | QPSK | S_0@0 |
| 6 | 100 | 100 | 5 |  |  | QPSK | P_18@82 | QPSK | S_0@0 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-3. Note 2: The carrier centre frequency of PCC in the UL operating band is configured closer to the DL operating band. |  |  |  |  |  |  |  |  |  |

6.5.2A.3.3.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.5.2A.3.3.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.5.2A.3.3.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is 3.2 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or 3.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

6.	Measure In-band emission on PCC using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test.

7.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is -26.8 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -26.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

8.	Measure In-band emission on PCC using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test.

9.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is to -36.8 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -36.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

10.	Measure In-band emission on PCC using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test.

6.5.2A.3.3.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

6.5.2A.3.3.5	Test requirement

Each of the 20 In-band emissions results, derived in Annex E.4.3 shall not exceed the corresponding values in Table 6.5.2A.3.3.5-1.

Table 6.5.2A.3.3.5-1: Test requirements for in-band emissions

| Parameter description | Unit | Limit (Note 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | +0.8 |  | Any non-allocated (Note 2) |
| IQ Image | dB | -27.2 | Image frequencies when carrier centre frequency < 1 GHz and Output power > 10 dBm | Image frequencies (Notes 2, 3) |
|  |  | -24.2 | Image frequencies when carrier centre frequency < 1 GHz and Output power ≤ 10 dBm |  |
|  |  | -24.2 | Image frequencies when carrier centre frequency ≥ 1 GHz |  |
| Carrier leakage | dBc | -27.2 | Output power > 10 dBm and carrier centre frequency < 1 GHz | Carrier frequency (Notes 4, 5) |
|  |  | -24.2 | Output power > 10 dBm and carrier centre frequency ≥ 1 GHz |  |
|  |  | -24.2 | 0 dBm ≤ Output power ≤10 dBm |  |
|  |  | -19.2 | -30 dBm ≤ Output power ≤ 0 dBm |  |
|  |  | -9.2 | -40 dBm Output power < -30 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB - 29.2 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in Note 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the centre carrier frequency, but excluding any allocated RBs. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit are those that are enclosed in the RBs containing the DC frequency if is odd, or in the two RBs immediately adjacent to the DC frequency if is even, but excluding any allocated RB. NOTE 6: is the Transmission Bandwidth (see Figure 5.4.2-1). NOTE 7: is the Transmission Bandwidth Configuration (see Figure 5.4.2-1). NOTE 8: is the limit specified in Table 6.5.2.1.3-1 for the modulation format used in the allocated RBs. NOTE 9: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. or for the first adjacent RB outside of the allocated bandwidth. NOTE 10: is the transmitted power per 180 kHz in allocated RBs, measured in dBm. |  |  |  |  |

##### 6.5.2A.3.4	In-band emissions for non allocated RB for CA (3UL CA)

6.5.2A.3.4.1	Test Purpose

Same test purpose as in clause 6.5.2A.3.1.1.

6.5.2A.3.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 13 and forward that support intra-band contiguous or inter-band 3DL CA and 3UL CA.

6.5.2A.3.4.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5.2A.3.0.

6.5.2A.3.4.4	Test description

6.5.2A.3.4.4.1		Initial conditions

Same as in clause 6.5.2A.3.1.4.1 with the following exceptions:

-	Instead of Table 5.4.2A.1-1 use Table 5.4.2A.1-2 for CA configurations with two bands or Table 5.4.2A.1-2a for CA configurations with three bands.

-	Instead of Table 6.5.2A.3.1.4.1-1  use Table 6.5.2A.3.4.4.1-1 to 6.5.2A.3.4.4.1-2.

-	Instead of clause 6.5.2A.3.1.4.3  use clause 6.5.2A.3.4.4.3

Table 6.5.2A.3.4.4.1-1: Test Configuration Table for intra-band contiguous CA

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Low range, High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  | Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |
|  | PCC NRB Note 1 | SCCs NRB Note 1 | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC1 & SCC2 RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 50 | 50 | N/A | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 | - |
| 2 | 50 | 50 |  | QPSK | 12 | P_12@38 | S_0@0 | S_0@0 | - |
| 3 | 50 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | - |
| 4 | 50 | 25 |  | QPSK | 8 | P_8@42 | S_0@0 | S_0@0 | - |
| 5 | 25 | 50 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | - |
| 6 | 25 | 50 |  | QPSK | 8 | P_8@17 | S_0@0 | S_0@0 | - |
| 7 | 75 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | S_0@0 | - |
| 8 | 75 | 75 |  | QPSK | 16 | P_16@59 | S_0@0 | S_0@0 | - |
| 9 | 100 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | - |
| 10 | 100 | 25 |  | QPSK | 8 | P_8@92 | S_0@0 | S_0@0 | - |
| 11 | 25 | 100 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | - |
| 12 | 25 | 100 |  | QPSK | 8 | P_8@17 | S_0@0 | S_0@0 | - |
| 13 | 100 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 | - |
| 14 | 100 | 50 |  | QPSK | 12 | P_12@88 | S_0@0 | S_0@0 | - |
| 15 | 50 | 100 |  | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 | - |
| 16 | 50 | 100 |  | QPSK | 12 | P_12@38 | S_0@0 | S_0@0 | - |
| 17 | 100 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | S_0@0 | - |
| 18 | 100 | 75 |  | QPSK | 16 | P_16@84 | S_0@0 | S_0@0 | - |
| 19 | 75 | 100 |  | QPSK | 16 | P_16@0 | S_0@0 | S_0@0 | - |
| 20 | 75 | 100 |  | QPSK | 16 | P_16@59 | S_0@0 | S_0@0 | - |
| 21 | 100 | 100 |  | QPSK | 18 | P_18@0 | S_0@0 | S_0@0 | - |
| 22 | 100 | 100 |  | QPSK | 18 | P_18@82 | S_0@0 | S_0@0 | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |  |

Table 6.5.2A.3.4.4.1-2: Test Configuration Table for inter-band CA

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  | Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |
|  | PCC NRB Note 1 | SCCs NRB Note 1 | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC1 & SCC2 RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 6 | 25 | N/A | QPSK | 1 | P_1@0 | S_0@0 | S_0@0 | - |
| 2 | 6 | 25 |  | QPSK | 1 | P_1@5 | S_0@0 | S_0@0 | - |
| 3 | 6 | 50 |  | QPSK | 1 | P_1@0 | S_0@0 | S_0@0 | - |
| 4 | 6 | 50 |  | QPSK | 1 | P_1@5 | S_0@0 | S_0@0 | - |
| 5 | 15 | 25 |  | QPSK | 4 | P_4@0 | S_0@0 | S_0@0 | - |
| 6 | 15 | 25 |  | QPSK | 4 | P_4@11 | S_0@0 | S_0@0 | - |
| 7 | 25 | 6 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | - |
| 8 | 25 | 6 |  | QPSK | 8 | P_8@17 | S_0@0 | S_0@0 | - |
| 9 | 25 | 15 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | - |
| 10 | 25 | 15 |  | QPSK | 8 | P_8@17 | S_0@0 | S_0@0 | - |
| 11 | 25 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | - |
| 12 | 25 | 25 |  | QPSK | 8 | P_8@17 | S_0@0 | S_0@0 | - |
| 13 | 25 | 50 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | - |
| 14 | 25 | 50 |  | QPSK | 8 | P_8@17 | S_0@0 | S_0@0 | - |
| 15 | 50 | 6 |  | QPSK | 12 | P_12@38 | S_0@0 | S_0@0 | - |
| 16 | 50 | 25 |  | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 | - |
| 17 | 50 | 25 |  | QPSK | 12 | P_12@38 | S_0@0 | S_0@0 | - |
| 18 | 50 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 | - |
| 19 | 50 | 50 |  | QPSK | 12 | P_12@38 | S_0@0 | S_0@0 | - |
| 20 | 50 | 100 |  | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 | - |
| 21 | 50 | 100 |  | QPSK | 12 | P_12@38 | S_0@0 | S_0@0 | - |
| 22 | 75 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | S_0@0 | - |
| 23 | 75 | 75 |  | QPSK | 16 | P_16@59 | S_0@0 | S_0@0 | - |
| 24 | 75 | 100 |  | QPSK | 16 | P_16@0 | S_0@0 | S_0@0 | - |
| 25 | 75 | 100 |  | QPSK | 16 | P_16@59 | S_0@0 | S_0@0 | - |
| 26 | 100 | 50 |  | QPSK | 18 | P_18@0 | S_0@0 | S_0@0 | - |
| 27 | 100 | 50 |  | QPSK | 18 | P_18@82 | S_0@0 | S_0@0 | - |
| 28 | 100 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | S_0@0 | - |
| 29 | 100 | 75 |  | QPSK | 16 | P_16@84 | S_0@0 | S_0@0 | - |
| 30 | 100 | 100 |  | QPSK | 18 | P_18@0 | S_0@0 | S_0@0 | - |
| 31 | 100 | 100 |  | QPSK | 18 | P_18@82 | S_0@0 | S_0@0 | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: The frequencies of PCC and SCC shall be switched and tested in each configuration. |  |  |  |  |  |  |  |  |  |

6.5.2A.3.4.4.2	Test procedure

1.	Configure SCCs according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCCs as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.5.2A.3.4.4.3.

3.	SS activates SCCs by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.5.2A.3.4.4.1-1 to 6.5.2A.3.4.4.1-2. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is 3.2 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or 3.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

6.	Measure carrier leakage on PCC using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test.

7.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is -26.8 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -26.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

8.	Measure carrier leakage on PCC using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test.

9.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is -36.8dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -36.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

10.	Measure carrier leakage on PCC using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test.

6.5.2A.3.4.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCCs configuration there are no additional message contents.

6.5.2A.3.4.5	Test requirement

Each of the 20 In-band emissions results, derived in Annex E.4.3 shall not exceed the corresponding values in Table 6.5.2A.3.4.5-1 to 6.5.2A.3.4.5-3.

Table 6.5.2A.3.4.5-1: Test requirements for in-band emissions (allocated component carrier) – intra-band contiguous CA

| Parameter description | Unit | Limit (Note 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | +0.8 |  | Any non-allocated (Note 2) |
| IQ Image | dB | -24.2 |  | Image frequencies (Notes 2, 3) |
| Carrier leakage | dBc | -24.2 | Output power f ≤ 3.0GHz: 3.2dBm ±3.2dB 3.0GHz < f ≤ 4.2GHz: 3.5 dBm ±3.5dB | Carrier frequency (Notes 4, 5) |
|  |  | -19.2 | Output power f ≤ 3.0GHz: -26.8 dBm ±3.2dB 3.0GHz < f ≤ 4.2GHz: -26.5 dBm ±3.5dB |  |
|  |  | -9.2 | Output power f ≤ 3.0GHz: -36.8 dBm ±3.2dB 3.0GHz < f ≤ 4.2GHz: -36.5 dBm ±3.5dB |  |
| Note 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the test requirement is calculated as the higher of PRB – 29.2 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in Note 10. Note 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. Note 3: The applicable frequencies for this limit are not known due to unknown LO frequency. See Note 3 in Table 6.5.2A.3.0-1. Note 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. Note 5: The applicable frequencies for this limit are not known due to unknown LO frequency. See Note 4 in Table 6.5.2A.3.0-1. Note 6: is the Transmission Bandwidth (see Figure 5.4.2-1). Note 7: is the Transmission Bandwidth Configuration (see Figure 5.4.2-1) of the component carrier with RBs allocated. Note 8: is the limit specified in Table 6.5.2.1.3-1 for the modulation format used in the allocated RBs. Note 9: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. or for the first adjacent RB outside of the allocated bandwidth). Note 10: is the transmitted power per 180 kHz in allocated RBs, measured in dBm. |  |  |  |  |

Table 6.5.2A.3.4.5-2: Test requirements for in-band emissions (not allocated component carrier) – intra-band contiguous CA

| Para-meter | Unit | Meas BW Note 1 | Limit |  | remark | Applicable Frequencies |
| --- | --- | --- | --- | --- | --- | --- |
| General | dB | BW of 1 RB (180KHz rectangular) |  |  | The reference value is the average power per allocated RB in the allocated component carrier | Any RB in the non allocated component carrier. The frequency raster of the RBs is derived when this component carrier is allocated with RBs |
| IQ Image | dB | BW of 1 RB (180KHz rectangular) | -24.2 Note 2 |  | The reference value is the average power per allocated RB in the allocated component carrier | The frequencies of thecontiguous non-allocated RBs are unknown. The frequency raster of the RBs is derived when this component carrier is allocated with RBs |
| Carrier leakage | dBc | BW of 1 RB (180KHz rectangular) | Note 3 |  | The reference value is the total power of the allocated RBs in the allocated component carrier | The frequencies of the up to 2 non-allocated RBs are unknown. The frequency raster of the RBs is derived when this component carrier is allocated with RBs |
|  |  |  | -24.2 | Output power > 0 dBm |  |  |
|  |  |  | -19.2 | -30 dBm ≤ Output power ≤ 0 dBm |  |  |
|  |  |  | -9.2 | -40 dBm Output power < -30 dBm |  |  |
| Note1: Resolution BWs smaller than the measurement BW may be integrated to achieve the measurement bandwidth. Note 2: Exceptions to the general limit are allowed for up to +1 RBs within a contiguous width of +1 non-allocated RBs. Note 3: Exceptions to the general limit are allowed for up to two contiguous non-allocated RBs. Note 4: Note s1, 6, 7, 8, 9, 10 from Table 6.5.2A.3.0-1 apply for Table 6.5.2A.3.0-2 as well. Note 5: for measured non-allocated RB in the non allocated component carrier may take non-integer values when the carrier spacing between the CCs is not a multiple of RB. |  |  |  |  |  |  |

Table 6.5.2A.3.4.5-3: Test requirements for in-band emissions – inter-band CA

| Parameter description | Unit | Limit (Note 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | +0.8 |  | Any non-allocated (Note 2) |
| IQ Image | dB | -27.2 | Image frequencies when carrier centre frequency < 1 GHz and Output power > 10 dBm | Image frequencies (Notes 2, 3) |
|  |  | -24.2 | Image frequencies when carrier centre frequency < 1 GHz and Output power ≤ 10 dBm |  |
|  |  | -24.2 | Image frequencies when carrier centre frequency ≥ 1 GHz |  |
| Carrier leakage | dBc | -27.2 | Output power > 10 dBm and carrier centre frequency < 1 GHz | Carrier frequency (Notes 4, 5) |
|  |  | -24.2 | Output power > 10 dBm and carrier centre frequency ≥ 1 GHz |  |
|  |  | -24.2 | 0 dBm ≤ Output power ≤10 dBm |  |
|  |  | -19.2 | -30 dBm ≤ Output power ≤ 0 dBm |  |
|  |  | -9.2 | -40 dBm Output power < -30 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB - 29.2 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in Note 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the centre carrier frequency, but excluding any allocated RBs. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit are those that are enclosed in the RBs containing the DC frequency if is odd, or in the two RBs immediately adjacent to the DC frequency if is even, but excluding any allocated RB. NOTE 6: is the Transmission Bandwidth (see Figure 5.4.2-1). NOTE 7: is the Transmission Bandwidth Configuration (see Figure 5.4.2-1). NOTE 8: is the limit specified in Table 6.5.2.1.1-1 for the modulation format used in the allocated RBs. NOTE 9: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. or for the first adjacent RB outside of the allocated bandwidth. NOTE 10: is the transmitted power per 180 kHz in allocated RBs, measured in dBm. |  |  |  |  |

##### 6.5.2A.3.5	In-band emissions for non allocated RB for CA (3UL CA)

6.5.2A.3.5.1	Test Purpose

Same test purpose as in clause 6.5.2A.3.1.1.

6.5.2A.3.5.2	Test applicability

This test case applies to all types of E-UTRA UE release 11 and forward that support 4DL CA and 4UL CA.

6.5.2A.3.5.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5.2A.3.0.

6.5.2A.3.5.4	Test description

6.5.2A.3.5.4.1		Initial conditions

Same as in clause 6.5.2A.3.1.4.1 with the following exceptions:

-	Instead of Table 5.4.2A.1-1 use Table 5.4.2A.1

-	Instead of Table 6.5.2A.3.1.4.1-1  use Table 6.5.2A.3.5.4.1-1 to 6.5.2A.3.5.4.1-2.

-	Instead of clause 6.5.2A.3.1.4.3  use clause 6.5.2A.3.5.4.3

Table 6.5.2A.3.5.4.1-1: Test Configuration Table for intra-band contiguous CA

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Low range, High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  | Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |
|  | PCC NRB Note 1 | SCCs NRB Note 1 | PCC & SCCs RB allocation |  | NRB_alloc | PCC & SCC1 & SCC2 & SCC3 RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 50 | 50 | N/A | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 | S_0@0 |
| 2 | 50 | 50 |  | QPSK | 12 | P_12@38 | S_0@0 | S_0@0 | S_0@0 |
| 3 | 50 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | S_0@0 |
| 4 | 50 | 25 |  | QPSK | 8 | P_8@42 | S_0@0 | S_0@0 | S_0@0 |
| 5 | 25 | 50 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | S_0@0 |
| 6 | 25 | 50 |  | QPSK | 8 | P_8@17 | S_0@0 | S_0@0 | S_0@0 |
| 7 | 75 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | S_0@0 | S_0@0 |
| 8 | 75 | 75 |  | QPSK | 16 | P_16@59 | S_0@0 | S_0@0 | S_0@0 |
| 9 | 100 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | S_0@0 |
| 10 | 100 | 25 |  | QPSK | 8 | P_8@92 | S_0@0 | S_0@0 | S_0@0 |
| 11 | 25 | 100 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | S_0@0 |
| 12 | 25 | 100 |  | QPSK | 8 | P_8@17 | S_0@0 | S_0@0 | S_0@0 |
| 13 | 100 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 | S_0@0 |
| 14 | 100 | 50 |  | QPSK | 12 | P_12@88 | S_0@0 | S_0@0 | S_0@0 |
| 15 | 50 | 100 |  | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 | S_0@0 |
| 16 | 50 | 100 |  | QPSK | 12 | P_12@38 | S_0@0 | S_0@0 | S_0@0 |
| 17 | 100 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | S_0@0 | S_0@0 |
| 18 | 100 | 75 |  | QPSK | 16 | P_16@84 | S_0@0 | S_0@0 | S_0@0 |
| 19 | 75 | 100 |  | QPSK | 16 | P_16@0 | S_0@0 | S_0@0 | S_0@0 |
| 20 | 75 | 100 |  | QPSK | 16 | P_16@59 | S_0@0 | S_0@0 | S_0@0 |
| 21 | 100 | 100 |  | QPSK | 18 | P_18@0 | S_0@0 | S_0@0 | S_0@0 |
| 22 | 100 | 100 |  | QPSK | 18 | P_18@82 | S_0@0 | S_0@0 | S_0@0 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |  |

Table 6.5.2A.3.5.4.1-2: Test Configuration Table for inter-band CA

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  | Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |
|  | PCC NRB Note 1 | SCCs NRB Note 1 | PCC & SCCs RB allocation |  | NRB_alloc | PCC & SCC1 & SCC2 & SCC3 RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 6 | 25 | N/A | QPSK | 1 | P_1@0 | S_0@0 | S_0@0 | S_0@0 |
| 2 | 6 | 25 |  | QPSK | 1 | P_1@5 | S_0@0 | S_0@0 | S_0@0 |
| 3 | 6 | 50 |  | QPSK | 1 | P_1@0 | S_0@0 | S_0@0 | S_0@0 |
| 4 | 6 | 50 |  | QPSK | 1 | P_1@5 | S_0@0 | S_0@0 | S_0@0 |
| 5 | 15 | 25 |  | QPSK | 4 | P_4@0 | S_0@0 | S_0@0 | S_0@0 |
| 6 | 15 | 25 |  | QPSK | 4 | P_4@11 | S_0@0 | S_0@0 | S_0@0 |
| 7 | 25 | 6 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | S_0@0 |
| 8 | 25 | 6 |  | QPSK | 8 | P_8@17 | S_0@0 | S_0@0 | S_0@0 |
| 9 | 25 | 15 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | S_0@0 |
| 10 | 25 | 15 |  | QPSK | 8 | P_8@17 | S_0@0 | S_0@0 | S_0@0 |
| 11 | 25 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | S_0@0 |
| 12 | 25 | 25 |  | QPSK | 8 | P_8@17 | S_0@0 | S_0@0 | S_0@0 |
| 13 | 25 | 50 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | S_0@0 |
| 14 | 25 | 50 |  | QPSK | 8 | P_8@17 | S_0@0 | S_0@0 | S_0@0 |
| 15 | 50 | 6 |  | QPSK | 12 | P_12@38 | S_0@0 | S_0@0 | S_0@0 |
| 16 | 50 | 25 |  | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 | S_0@0 |
| 17 | 50 | 25 |  | QPSK | 12 | P_12@38 | S_0@0 | S_0@0 | S_0@0 |
| 18 | 50 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 | S_0@0 |
| 19 | 50 | 50 |  | QPSK | 12 | P_12@38 | S_0@0 | S_0@0 | S_0@0 |
| 20 | 50 | 100 |  | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 | S_0@0 |
| 21 | 50 | 100 |  | QPSK | 12 | P_12@38 | S_0@0 | S_0@0 | S_0@0 |
| 22 | 75 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | S_0@0 | S_0@0 |
| 23 | 75 | 75 |  | QPSK | 16 | P_16@59 | S_0@0 | S_0@0 | S_0@0 |
| 24 | 75 | 100 |  | QPSK | 16 | P_16@0 | S_0@0 | S_0@0 | S_0@0 |
| 25 | 75 | 100 |  | QPSK | 16 | P_16@59 | S_0@0 | S_0@0 | S_0@0 |
| 26 | 100 | 50 |  | QPSK | 18 | P_18@0 | S_0@0 | S_0@0 | S_0@0 |
| 27 | 100 | 50 |  | QPSK | 18 | P_18@82 | S_0@0 | S_0@0 | S_0@0 |
| 28 | 100 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | S_0@0 | S_0@0 |
| 29 | 100 | 75 |  | QPSK | 16 | P_16@84 | S_0@0 | S_0@0 | S_0@0 |
| 30 | 100 | 100 |  | QPSK | 18 | P_18@0 | S_0@0 | S_0@0 | S_0@0 |
| 31 | 100 | 100 |  | QPSK | 18 | P_18@82 | S_0@0 | S_0@0 | S_0@0 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: The frequencies of PCC and SCC shall be switched and tested in each configuration. |  |  |  |  |  |  |  |  |  |

6.5.2A.3.5.4.2	Test procedure

1.	Configure SCCs according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCCs as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.5.2A.3.5.4.3.

3.	SS activates SCCs by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.5.2A.3.5.4.1-1 to 6.5.2A.3.5.4.1-2. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is 3.2 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or 3.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

6.	Measure carrier leakage on PCC using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test.

7.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is -26.8 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -26.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

8.	Measure carrier leakage on PCC using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test.

9.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is -36.8dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -36.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

10.	Measure carrier leakage on PCC using Global In-Channel Tx-Test (Annex E). For TDD slots with transient periods are not under test.

6.5.2A.3.5.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCCs configuration there are no additional message contents.

6.5.2A.3.5.5	Test requirement

Each of the 20 In-band emissions results, derived in Annex E.4.3 shall not exceed the corresponding values in Table 6.5.2A.3.5.5-1 to 6.5.2A.3.5.5-3.

Table 6.5.2A.3.5.5-1: Test requirements for in-band emissions (allocated component carrier) – intra-band contiguous CA

| Parameter description | Unit | Limit (Note 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | +0.8 |  | Any non-allocated (Note 2) |
| IQ Image | dB | -24.2 |  | Image frequencies (Notes 2, 3) |
| Carrier leakage | dBc | -24.2 | Output power f ≤ 3.0GHz: 3.2dBm ±3.2dB 3.0GHz < f ≤ 4.2GHz: 3.5 dBm ±3.5dB | Carrier frequency (Notes 4, 5) |
|  |  | -19.2 | Output power f ≤ 3.0GHz: -26.8 dBm ±3.2dB 3.0GHz < f ≤ 4.2GHz: -26.5 dBm ±3.5dB |  |
|  |  | -9.2 | Output power f ≤ 3.0GHz: -36.8 dBm ±3.2dB 3.0GHz < f ≤ 4.2GHz: -36.5 dBm ±3.5dB |  |
| Note 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the test requirement is calculated as the higher of PRB – 29.2 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in Note 10. Note 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. Note 3: The applicable frequencies for this limit are not known due to unknown LO frequency. See Note 3 in Table 6.5.2A.3.0-1. Note 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. Note 5: The applicable frequencies for this limit are not known due to unknown LO frequency. See Note 4 in Table 6.5.2A.3.0-1. Note 6: is the Transmission Bandwidth (see Figure 5.4.2-1). Note 7: is the Transmission Bandwidth Configuration (see Figure 5.4.2-1) of the component carrier with RBs allocated. Note 8: is the limit specified in Table 6.5.2.1.3-1 for the modulation format used in the allocated RBs. Note 9: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. or for the first adjacent RB outside of the allocated bandwidth). Note 10: is the transmitted power per 180 kHz in allocated RBs, measured in dBm. |  |  |  |  |

Table 6.5.2A.3.5.5-2: Test requirements for in-band emissions (not allocated component carrier) – intra-band contiguous CA

| Para-meter | Unit | Meas BW Note 1 | Limit |  | remark | Applicable Frequencies |
| --- | --- | --- | --- | --- | --- | --- |
| General | dB | BW of 1 RB (180KHz rectangular) |  |  | The reference value is the average power per allocated RB in the allocated component carrier | Any RB in the non allocated component carrier. The frequency raster of the RBs is derived when this component carrier is allocated with RBs |
| IQ Image | dB | BW of 1 RB (180KHz rectangular) | -24.2 Note 2 |  | The reference value is the average power per allocated RB in the allocated component carrier | The frequencies of thecontiguous non-allocated RBs are unknown. The frequency raster of the RBs is derived when this component carrier is allocated with RBs |
| Carrier leakage | dBc | BW of 1 RB (180KHz rectangular) | Note 3 |  | The reference value is the total power of the allocated RBs in the allocated component carrier | The frequencies of the up to 2 non-allocated RBs are unknown. The frequency raster of the RBs is derived when this component carrier is allocated with RBs |
|  |  |  | -24.2 | Output power > 0 dBm |  |  |
|  |  |  | -19.2 | -30 dBm ≤ Output power ≤ 0 dBm |  |  |
|  |  |  | -9.2 | -40 dBm Output power < -30 dBm |  |  |
| Note1: Resolution BWs smaller than the measurement BW may be integrated to achieve the measurement bandwidth. Note 2: Exceptions to the general limit are allowed for up to +1 RBs within a contiguous width of +1 non-allocated RBs. Note 3: Exceptions to the general limit are allowed for up to two contiguous non-allocated RBs. Note 4: Note s1, 6, 7, 8, 9, 10 from Table 6.5.2A.3.0-1 apply for Table 6.5.2A.3.0-2 as well. Note 5: for measured non-allocated RB in the non allocated component carrier may take non-integer values when the carrier spacing between the CCs is not a multiple of RB. |  |  |  |  |  |  |

Table 6.5.2A.3.5.5-3: Test requirements for in-band emissions – inter-band CA

| Parameter description | Unit | Limit (Note 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | +0.8 |  | Any non-allocated (Note 2) |
| IQ Image | dB | -27.2 | Image frequencies when carrier centre frequency < 1 GHz and Output power > 10 dBm | Image frequencies (Notes 2, 3) |
|  |  | -24.2 | Image frequencies when carrier centre frequency < 1 GHz and Output power ≤ 10 dBm |  |
|  |  | -24.2 | Image frequencies when carrier centre frequency ≥ 1 GHz |  |
| Carrier leakage | dBc | -27.2 | Output power > 10 dBm and carrier centre frequency < 1 GHz | Carrier frequency (Notes 4, 5) |
|  |  | -24.2 | Output power > 10 dBm and carrier centre frequency ≥ 1 GHz |  |
|  |  | -24.2 | 0 dBm ≤ Output power ≤10 dBm |  |
|  |  | -19.2 | -30 dBm ≤ Output power ≤ 0 dBm |  |
|  |  | -9.2 | -40 dBm Output power < -30 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB - 29.2 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in Note 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the centre carrier frequency, but excluding any allocated RBs. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit are those that are enclosed in the RBs containing the DC frequency if is odd, or in the two RBs immediately adjacent to the DC frequency if is even, but excluding any allocated RB. NOTE 6: is the Transmission Bandwidth (see Figure 5.4.2-1). NOTE 7: is the Transmission Bandwidth Configuration (see Figure 5.4.2-1). NOTE 8: is the limit specified in Table 6.5.2.1.1-1 for the modulation format used in the allocated RBs. NOTE 9: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. or for the first adjacent RB outside of the allocated bandwidth. NOTE 10: is the transmitted power per 180 kHz in allocated RBs, measured in dBm. |  |  |  |  |

### 6.5.2B	Transmit modulation for UL- MIMO

#### 6.5.2B.1	Error Vector Magnitude (EVM) for UL- MIMO

##### 6.5.2B.1.1	Test Purpose

The Error Vector Magnitude is a measure of the difference between the reference waveform and the measured waveform. This difference is called the error vector. Before calculating the EVM the measured waveform is corrected by the sample timing offset and RF frequency offset. Then the carrier leakage shall be removed from the measured waveform before calculating the EVM.

The measured waveform is further modified by selecting the absolute phase and absolute amplitude of the Tx chain. The EVM result is defined after the front-end IDFT as the square root of the ratio of the mean error vector power to the mean reference power expressed as a %.

When the PUSCH or PUCCH transmission slot is shortened due to multiplexing with SRS, the EVM measurement interval is reduced by one symbol, accordingly. The PUSCH or PUCCH EVM measurement interval is also reduced when the mean power, modulation or allocation between slots is expected to change. In the case of PUSCH transmission, the measurement interval is reduced by a time interval equal to the sum of 5 μs and the applicable exclusion period defined in subclause 6.3.4, adjacent to the boundary where the power change is expected to occur. The PUSCH exclusion period is applied to the signal obtained after the front-end IDFT. In the case of PUCCH transmission, the PUCCH EVM measurement interval is reduced by one symbol adjacent to the slot boundary.

##### 6.5.2B.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 10 and forward that support UL-MIMO.

##### 6.5.2B.1.3	Minimum conformance requirements

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the Error Vector Magnitude requirements specified in Table 6.5.2.1.1-1 which is defined in sub-clause 6.5.2.1 apply to each transmit antenna connector with the UL-MIMO configurations specified in Table 6.2.2B-2.

The normative reference for this requirement is TS 36.101 [2] clause 6.5.2B.1.1.

##### 6.5.2B.1.4	Test description

6.5.2B.1.4.1		Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.5.2B.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.2B.1.4.1-1: Test Configuration Table for PUSCH

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  | NC |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  | See Table 6.5.1.4.1-1 |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  | See Table 6.5.1.4.1-1 |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
|  | Downlink Configuration | Uplink Configuration |  |  |
| Ch BW | N/A for PUSCH EVM testing | Mod'n | RB allocation |  |
|  |  |  | FDD | TDD |
| 1.4MHz |  | QPSK | 6 | 6 |
| 1.4MHz |  | QPSK | 1 | 1 |
| 1.4MHz |  | 16QAM | 6 | 6 |
| 1.4MHz |  | 16QAM | 1 | 1 |
| 3MHz |  | QPSK | 15 | 15 |
| 3MHz |  | QPSK | 4 | 4 |
| 3MHz |  | 16QAM | 15 | 15 |
| 3MHz |  | 16QAM | 4 | 4 |
| 5MHz |  | QPSK | 25 | 25 |
| 5MHz |  | QPSK | 8 | 8 |
| 5MHz |  | 16QAM | 25 | 25 |
| 5MHz |  | 16QAM | 8 | 8 |
| 10MHz |  | QPSK | 50 | 50 |
| 10MHz |  | QPSK | 12 | 12 |
| 10MHz |  | 16QAM | 50 (Note 3) | 50 (Note 3) |
| 10MHz |  | 16QAM | 12 | 12 |
| 15MHz |  | QPSK | 75 | 75 |
| 15MHz |  | QPSK | 16 | 16 |
| 15MHz |  | 16QAM | 75 (Note 3) | 75 (Note 3) |
| 15MHz |  | 16QAM | 16 | 16 |
| 20MHz |  | QPSK | 100 | 100 |
| 20MHz |  | QPSK | 18 | 18 |
| 20MHz |  | 16QAM | 100 (Note 3) | 100 (Note 3) |
| 20MHz |  | 16QAM | 18 | 18 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: For partial RB allocation, the RBstart shall be RB #0 and RB# (max+1 - RB allocation) of the channel bandwidth. Note 3: Applies only for UE-Categories [FFS]. |  |  |  |  |

Table 6.5.2B.1.4.1-2: Test Configuration Table for PUCCH

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  | NC |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 |  |  | See Table 6.5.1.4.1-1 |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | See Table 6.5.1.4.1-1 |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |
| Ch BW | Mod'n | RB allocation |  | FDD: PUCCH format = Format 1a TDD: PUCCH format = Format 1a / 1b |
|  |  | FDD | TDD |  |
| 1.4MHz | QPSK | 3 | 3 |  |
| 3MHz | QPSK | 4 | 4 |  |
| 5MHz | QPSK | 8 | 8 |  |
| 10MHz | QPSK | 16 | 16 |  |
| 15MHz | QPSK | 25 | 25 |  |
| 20MHz | QPSK | 30 | 30 |  |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. |  |  |  |  |

1.	Connect the SS to the UE to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.28.

2.	The parameter settings for the cell are set up according to TS 36.508[7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to in Table 6.5.2B.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.5.2B.1.4.3.

6.5.2B.1.4.2	Test procedure

Test procedure for PUSCH:

1.1.SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 4 for C_RNTI to schedule the UL RMC according to Table 6.5.2B.1.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

1.2	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

1.3	Measure the EVM and  using Global In-Channel Tx-Test (Annex E) for each of transmit antenna of the UE.

1.4	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is –36.8dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -36.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

1.5	Measure the EVM and  using Global In-Channel Tx-Test (Annex E) for each of transmit antenna of the UE.

Test procedure for PUCCH:

2.1	PUCCH is set according to Table 6.5.2B.1.4.1-2.

2.2	SS transmits PDSCH via PDCCH DCI format 1A for C_RNTI to transmit the DL RMC according to Table 6.5.2B.1.4.1-2. The SS sends downlink MAC padding bits on the DL RMC. The transmission of PDSCH will make the UE send uplink ACK/NACK using PUCCH. There is no PUSCH transmission.

2.3	SS send appropriate TPC commands for PUCCH to the UE until the UE transmit PUCCH at PUMAX level.

2.4	Measure PUCCH EVM using Global In-Channel Tx-Test (Annex E) for each of transmit antenna of the UE.

2.5	Send the appropriate TPC commands for PUCCH to the UE until the UE transmits PUCCH at -36.8dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -36.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

2.6	Measure PUCCH EVM using Global In-Channel Tx-Test (Annex E) for each of transmit antenna of the UE.

6.5.2B.1.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6 with the following exceptions:

Table 6.5.2B.1.4.3-1: PUCCH-ConfigDedicated-v1020-DEFAULT

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-9A: PUCCH-ConfigDedicated-v1020-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-ConfigDedicated-v1020 ::= SEQUENCE { |  |  |  |
| twoAntennaPortActivatedPUCCH-Format1a1b-r10 | true |  |  |
| } |  |  |  |

##### 6.5.2B.1.5	Test requirement

The requirements apply to each transmit antenna connector.

The PUSCH EVM derived in E.4.2 shall not exceed 17.5 % for QPSK and BPSK, 12.5% for 16 QAM.

The PUSCH derived in E.4.6.2 shall not exceed [17.5 %] when embedded with data symbols of QPSK and BPSK, [12,5%] for 16 QAM.

The PUCCH EVM and derived in E.5.9.2 shall not exceed 17.5 %.

#### 6.5.2B.2	Carrier leakage for UL-MIMO

##### 6.5.2B.2.1	Test Purpose

The purpose of this test is to exercise the transmitter of UE that support UL-MIMO to verify its modulation quality in terms of carrier leakage for UL-MIMO.

##### 6.5.2B.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 10 and forward that support UL-MIMO.

##### 6.5.2B.2.3	Minimum conformance requirements

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the Relative Carrier Leakage Power requirements specified in Table 6.5.2.2.3-1 which is defined in subclause 6.5.2.2 apply to each transmit antenna connector with the UL-MIMO configurations specified in Table 6.2.2B.3-2.

The normative reference for this requirement is TS 36.101 clause 6.5.2B.2

##### 6.5.2B.2.4	Test description

6.5.2B.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.5.2.2B.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.2.2B.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  | See Table 6.5.1.4.1-1 |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  | See Table 6.5.1.4.1-1 |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  | See Table 6.5.1.4.1-1 |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
|  | Downlink Configuration | Uplink Configuration |  |  |
| Ch BW | N/A for carrier leakage testing | Mod'n | RB allocation |  |
|  |  |  | FDD | TDD |
| 1.4MHz |  | QPSK | 1 | 1 |
| 3MHz |  | QPSK | 4 | 4 |
| 5MHz |  | QPSK | 8 | 8 |
| 10MHz |  | QPSK | 12 | 12 |
| 15MHz |  | QPSK | 16 | 16 |
| 20MHz |  | QPSK | 18 | 18 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: For partial RB allocation, the RBstart shall be RB #0 and RB# (max +1- RB allocation) of the channel bandwidth. |  |  |  |  |

1.	Connect the SS to the UE to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.28.

2.	The parameter settings for the cell are set up according to TS 36.508[7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to in Table 6.5.2B.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.5.2B.2.4.3.

6.5.2B.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 4 for C_RNTI to schedule the UL RMC according to Table 6.5.2B.2.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC

2.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is 3.2 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or 3.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

3.	Measure carrier leakage using Global In-Channel Tx-Test (Annex E) for each of transmit antenna of the UE. For TDD slots with transient periods are not under test.

4.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is -26.8 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -26.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

5.	Measure carrier leakage using Global In-Channel Tx-Test (Annex E) for each of transmit antenna of the UE. For TDD slots with transient periods are not under test.

6.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is -36.8dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -36.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

7.	Measure carrier leakage using Global In-Channel Tx-Test (Annex E) for each of transmit antenna of the UE. For TDD slots with transient periods are not under test

6.5.2B.2.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6.

##### 6.5.2B.2.5	Test requirement

The requirements apply to each transmit antenna connector.

Each of the 20 carrier leakage results, derived in Annex E.3.1, shall not exceed the values in table 6.5.2B.2.5-1

Table 6.5.2B.2.5-1: Test requirements for Relative Carrier Leakage Power

| LO Leakage | Parameters | Relative Limit (dBc) |
| --- | --- | --- |
|  | f ≤ 3.0GHz: 3.2 dBm ±3.2dB 3.0GHz < f ≤ 4.2GHz: 3.5 dBm ±3.5dB | 24.2 |
|  | f ≤ 3.0GHz: -26.8 dBm ±3.2dB 3.0GHz < f ≤ 4.2GHz: -26.5 dBm ±3.5dB | 19.2 |
|  | f ≤ 3.0GHz: -36.8dBm±3.2dB 3.0GHz < f ≤ 4.2GHz: -36.5 dBm ±3.5dB | 9.2 |

#### 6.5.2B.3	In-band emissions for non allocated RB for UL-MIMO

##### 6.5.2B.3.1	Test Purpose

The in-band emissions are a measure of the interference falling into the non-allocated resources blocks.

The in-band emission is defined as the average across 12 sub-carrier and as a function of the RB offset from the edge of the allocated UL transmission bandwidth. The in-band emission for UL-MIMO is measured as the ratio of the output power of UE that support UL-MIMO in a non–allocated RB to the output power of UE that support UL-MIMO in an allocated RB. The basic in-band emissions measurement interval is defined over one slot in the time domain. When the PUSCH or PUCCH transmission slot is shortened due to multiplexing with SRS, the in-band emissions measurement interval is reduced by one SC-FDMA symbol, accordingly.

##### 6.5.2B.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 10 and forward that support UL-MIMO.

##### 6.5.2B.3.3	Minimum conformance requirements

The relative in-band emission shall not exceed the values specified in Table 6.5.2B.3.3-1.

Table 6.5.2B.3.3-1: Minimum requirements for in-band emissions

| Parameter Description | Unit | Limit (Note 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB |  |  | Any non-allocated (Note 2) |
| IQ Image | dB | -25 |  | Image frequencies (Notes 2, 3) |
| Carrier leakage | dBc | -25 | Output power > 0 dBm | LO frequency (Notes 4, 5) |
|  |  | -20 | -30 dBm ≤ Output power ≤ 0 dBm |  |
|  |  | -10 | -40 dBm Output power < -30 dBm |  |
| Note 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB - 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in Note 10. Note 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. Note 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the centre carrier frequency, but excluding any allocated RBs. Note 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. Note 5: The applicable frequencies for this limit are those that are enclosed in the RBs containing the DC frequency if is odd, or in the two RBs immediately adjacent to the DC frequency if is even, but excluding any allocated RB. Note 6: is the Transmission Bandwidth (see Figure 5.4.2-1). Note 7: is the Transmission Bandwidth Configuration (see Figure 5.4.2-1). Note 8: is the limit specified in Table 6.5.2.1.1-1 for the modulation format used in the allocated RBs. Note 9: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. or for the first adjacent RB outside of the allocated bandwidth. Note 10: is the transmitted power per 180 kHz in allocated RBs, measured in dBm. |  |  |  |  |

The normative reference for this requirement is TS 36.101 [2] clause 6.5.2B.3.

The in-band emission is defined as the average across 12 sub-carrier and as a function of the RB offset from the edge of the allocated UL transmission bandwidth. The in-band emission is measured as the ratio of the UE output power in a non–allocated RB to the UE output power in an allocated RB. The basic in-band emissions measurement interval is defined over one slot in the time domain.

##### 6.5.2B.3.4	Test description

6.5.2B.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.5.2B.3.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.2B.3.4.1-1: Test Configuration Table for PUSCH

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  | See Table 6.5.1.4.1-1 |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  | See Table 6.5.1.4.1-1 |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  | See Table 6.5.1.4.1-1 |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
|  | Downlink Configuration | Uplink Configuration |  |  |
| Ch BW | N/A for in-band emissions testing | Mod'n | RB allocation |  |
|  |  |  | FDD | TDD |
| 1.4MHz |  | QPSK | 1 | 1 |
| 3MHz |  | QPSK | 4 | 4 |
| 5MHz |  | QPSK | 8 | 8 |
| 10MHz |  | QPSK | 12 | 12 |
| 15MHz |  | QPSK | 16 | 16 |
| 20MHz |  | QPSK | 18 | 18 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: For partial RB allocation, the starting resource block shall be RB #0 and RB# (max+1 - RB allocation) of the channel bandwidth. |  |  |  |  |

Table 6.5.2B.3.4.1-2: Test Configuration Table for PUCCH

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  | See Table 6.5.1.4.1-1 |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 |  |  | See Table 6.5.1.4.1-1 |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | See Table 6.5.1.4.1-1 |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |
| Ch BW | Mod'n | RB allocation |  | FDD: PUCCH format = Format 1a TDD: PUCCH format = Format 1a / 1b |
|  |  | FDD | TDD |  |
| 1.4MHz | QPSK | 3 | 3 |  |
| 3MHz | QPSK | 4 | 4 |  |
| 5MHz | QPSK | 8 | 8 |  |
| 10MHz | QPSK | 16 | 16 |  |
| 15MHz | QPSK | 25 | 25 |  |
| 20MHz | QPSK | 30 | 30 |  |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Figure A.28.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to in Table 6.5.2B.3.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.5.2B.3.4.3.

6.5.2B.3.4.2	Test procedure

Test procedure for PUSCH:

1.1	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 4 for C_RNTI to schedule the UL RMC according to Table 6.5.2B.3.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

1.2	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is 3.2 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or 3.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

1.3	Measure In-band emission using Global In-Channel Tx-Test (Annex E) for each of transmit antennas of the UE.

1.4	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is -26.8 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -36.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

1.5	Measure In-band emission using Global In-Channel Tx-Test (Annex E) for each of transmit antennas of the UE. For TDD slots with transient periods are not under test.

1.6	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is to -36.8 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -36.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

1,7	Measure In-band emission using Global In-Channel Tx-Test (Annex E) for each of transmit antennas of the UE. For TDD slots with transient periods are not under test.

Test procedure for PUCCH:

2.1	PUCCH is set according to Table 6.5.2B.3.4.1-2. SS transmits PDSCH via PDCCH DCI format 1A for C_RNTI to transmit the DL RMC according to Table 6.5.2B.3.4.1-2. The SS sends downlink MAC padding bits on the DL RMC. The transmission of PDSCH will make the UE send uplink ACK/NACK using PUCCH.

2.2	Send the appropriate TPC commands in the uplink scheduling information for PUCCH to the UE until UE output power is 3.2 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or 3.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

2.3	Measure In-band emission using Global In-Channel Tx-Test (Annex E) for each of transmit antennas of the UE.

2.4	Send the appropriate TPC commands for PUCCH in the uplink scheduling information to the UE until UE output power is -26.8 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -26.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

2.5	Measure In-band emission using Global In-Channel Tx-Test (Annex E) for each of transmit antennas of the UE.

2.6	Send the appropriate TPC commands for PUCCH in the uplink scheduling information to the UE until UE output power is to -36.8 dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -36.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

2.7	Measure In-band emission using Global In-Channel Tx-Test (Annex E) for each of transmit antennas of the UE.

6.5.2B.3.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6 with the following exceptions:

Table 6.5.2B.3.4.3-1: PUCCH-ConfigDedicated-v1020-DEFAULT

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-9A: PUCCH-ConfigDedicated-v1020-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-ConfigDedicated-v1020 ::= SEQUENCE { |  |  |  |
| twoAntennaPortActivatedPUCCH-Format1a1b-r10 | true |  |  |
| } |  |  |  |

##### 6.5.2B.3.5	Test requirement

The requirements apply to each transmit antenna connector.

Each of the 20 In-band emissions results, derived in Annex E.4.3 shall not exceed the corresponding values in Table 6.5.2B.3.5-1.

Table 6.5.2B.3.5-1: Test requirements for in-band emissions

| Parameter Description | Unit | Limit (Note 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | +0.8 |  | Any non-allocated (Note 2) |
| IQ Image | dB | -24.2 |  | Image frequencies (Notes 2, 3) |
| Carrier leakage | dBc | -24.2 | Output power > 0 dBm | LO frequency (Notes 4, 5) |
|  |  | -19.2 | -30 dBm ≤ Output power ≤ 0 dBm |  |
|  |  | -9.2 | -40 dBm Output power < -30 dBm |  |
| Note 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB - 29.2 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in Note 10. Note 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. Note 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the centre carrier frequency, but excluding any allocated RBs. Note 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. Note 5: The applicable frequencies for this limit are those that are enclosed in the RBs containing the DC frequency if is odd, or in the two RBs immediately adjacent to the DC frequency if is even, but excluding any allocated RB. Note 6: is the Transmission Bandwidth (see Figure 5.4.2-1). Note 7: is the Transmission Bandwidth Configuration (see Figure 5.4.2-1). Note 8: is the limit specified in Table 6.5.2.1.1-1 for the modulation format used in the allocated RBs. Note 9: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. or for the first adjacent RB outside of the allocated bandwidth. Note 10: is the transmitted power per 180 kHz in allocated RBs, measured in dBm. |  |  |  |  |

#### 6.5.2B.4	EVM equalizer spectrum flatness for UL-MIMO

##### 6.5.2B.4.1	Test Purpose

The zero-forcing equalizer correction applied in the EVM measurement process (as described in Annex E) must meet a spectrum flatness requirement for the EVM measurement to be valid. The EVM equalizer spectrum flatness is defined in terms of the maximum peak-to-peak ripple of the equalizer coefficients (dB) across the allocated uplink block variation in dB of the equalizer coefficients generated by the EVM measurement process. The EVM equalizer spectrum flatness requirement does not limit the correction applied to the signal in the EVM measurement process but for the EVM result to be valid, the equalizer correction that was applied must meet the EVM equalizer spectrum flatness minimum requirements. The basic measurement interval is the same as for EVM.

##### 6.5.2B.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 10 and forward that support UL-MIMO.

##### 6.5.2B.4.3	Minimum conformance requirements

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the EVM Equalizer Spectrum Flatness requirements specified in Table 6.5.2B.4.3-1 and Table 6.5.2B.4.3-2 which are defined in sub-clause 6.5.2B.4 apply to each transmit antenna connector with the UL-MIMO configurations specified in Table 6.2.2B.3-2.

The normative reference for this requirement is TS 36.101 clause 6.5.2B.4.

##### 6.5.2B.4.4	Test description

6.5.2B.4.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.5.2B.4.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.2B.4.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  | See Table 6.5.1.4.1-1 |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  | See Table 6.5.1.4.1-1 |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  | See Table 6.5.1.4.1-1 |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
|  | Downlink Configuration | Uplink Configuration |  |  |
| Ch BW | N/A for EVM equalizer spectrum flatness testing | Mod'n | RB allocation |  |
|  |  |  | FDD | TDD |
| 1.4MHz |  | QPSK | 6 | 6 |
| 3MHz |  | QPSK | 15 | 15 |
| 5MHz |  | QPSK | 25 | 25 |
| 10MHz |  | QPSK | 50 | 50 |
| 15MHz |  | QPSK | 75 | 75 |
| 20MHz |  | QPSK | 100 | 100 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in Table 5.4.2.1-1. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Figure A.28.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to in Table 6.5.2B.4.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.5.2B.4.4.3.

6.5.2B.4.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 4 for C_RNTI to schedule the UL RMC according to Table 6.5.2B.4.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure spectrum flatness using Global In-Channel Tx-Test (Annex E) for each of transmit antennas of the UE. For TDD slots with transient periods are not under test.

6.5.2B.4.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6.

##### 6.5.2B.4.5	Test requirement

The requirements apply to each transmit antenna connector.

Each of the 20 spectrum flatness functions, shall derive four ripple results in Annex E.4.4, The derived results shall not exceed the values in Figure 6.5.2B.4.5-1:

For normal conditions, the maximum ripple in Range 1 and Range 2 shall not exceed the values specified in Table 6.5.2B.4.5-1 and the following additional requirement: the relative difference between the maximum coefficient in Range 1 and the minimum coefficient in Range 2 must not be larger than 6.4 dB, and the relative difference between the maximum coefficient in Range 2 and the minimum coefficient in Range 1 must not be larger than 8.4 dB (see Figure 6.5.2B.4.5-1).

For extreme conditions, the maximum ripple in Range 1 and Range 2 shall not exceed the values specified in Table 6.5.2B.4.5-2 and the following additional requirement: the relative difference between the maximum coefficient in Range 1 and the minimum coefficient in Range 2 must not be larger than 7.4 dB, and the relative difference between the maximum coefficient in Range 2 and the minimum coefficient in Range 1 must not be larger than 11.4 dB (see Figure 6.5.2B.4.5-1).

Table 6.5.2B.4.5-1: Test requirements for EVM equalizer spectrum flatness (normal conditions)

| Frequency Range | Maximum Ripple [dB] |
| --- | --- |
| FUL_Meas – FUL_Low ≥ 3 MHz and FUL_High – FUL_Meas ≥ 3 MHz (Range 1) | 5.4 (p-p) |
| FUL_Meas – FUL_Low < 3 MHz or FUL_High – FUL_Meas < 3 MHz (Range 2) | 9.4 (p-p) |
| Note 1: FUL_Meas refers to the sub-carrier frequency for which the equalizer coefficient is evaluated Note 2: FUL_Low and FUL_High refer to each E-UTRA frequency band specified in Table 5.2-1 |  |

Table 6.5.2B.4.5-2: Test requirements for spectrum flatness (extreme conditions)

| FUL_Meas – FUL_Low ≥ 5 MHz and FUL_High – FUL_Meas ≥ 5 MHz (Range 1) | 5.4 (p-p) |
| --- | --- |
| FUL_Meas – FUL_Low < 5 MHz or FUL_High – FUL_Meas < 5 MHz (Range 2) | 13.4 (p-p) |
| Note 1: FUL_Meas refers to the sub-carrier frequency for which the equalizer coefficient is evaluated Note 2: FUL_Low and FUL_High refer to each E-UTRA frequency band specified in Table 5.2-1 |  |

Figure 6.5.2B.4.5-1: The limits for EVM equalizer spectrum flatness with the maximum allowed variation of the coefficients indicated (the ETC minimum requirement within brackets)

## 6.6	Output RF spectrum emissions

Unwanted emissions are divided into "Out-of-band emission" and "Spurious emissions" in 3GPP RF specifications. This notation is in line with ITU-R recommendations such as SM.329 ‎‎[2] and the Radio Regulations ‎[3].

ITU defines:

Out-of-band emission = Emission on a frequency or frequencies immediately outside the necessary bandwidth which results from the modulation process, but excluding spurious emissions.

Spurious emission = Emission on a frequency, or frequencies, which are outside the necessary bandwidth and the level of which may be reduced without affecting the corresponding transmission of information. Spurious emissions include harmonic emissions, parasitic emissions, intermodulation products and frequency conversion products but exclude out-of-band emissions.

Unwanted emissions = Consist of spurious emissions and out-of-band emissions.

The UE transmitter spectrum emission consists of the three components; the occupied bandwidth (channel bandwidth), the Out Of Band (OOB) emissions and the far out spurious emission domain.

Figure 6.6-1: Transmitter RF spectrum

### 6.6.1	Occupied bandwidth

#### 6.6.1.1	Test purpose

To verify that the UE occupied bandwidth for all transmission bandwidth configurations supported by the UE are less than their specific limits

#### 6.6.1.2	Test applicability

This test applies to all types of E-UTRA UE release 8 and forward.

#### 6.6.1.3	Minimum conformance requirements

Occupied bandwidth is a measure of the bandwidth containing 99 % of the total integrated mean power of the transmitted spectrum on the assigned channel. The occupied channel bandwidth for all transmission bandwidth configurations (Resources Blocks) should be less than the channel bandwidth specified in Table 6.6.1.3-1

Table 6.6.1.3-1: Occupied channel bandwidth

|  | Occupied channel bandwidth / channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Channel bandwidth [MHz] | 1.4 | 3 | 5 | 10 | 15 | 20 |

The normative reference for this requirement is TS 36.101 [2] clause 6.6.1.

#### 6.6.1.4	Test description

##### 6.6.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  | Normal |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 |  | Mid range |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  | All |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
|  | Downlink Configuration |  | Uplink Configuration |  |  |
| Ch BW | N/A for Occupied bandwidth |  | Mod'n | RB allocation |  |
|  |  |  |  | FDD | TDD |
| 1.4MHz |  |  | QPSK | 6 | 6 |
| 3MHz |  |  | QPSK | 15 | 15 |
| 5MHz |  |  | QPSK | 25 | 25 |
| 10MHz |  |  | QPSK | 50 | 50 |
| 15MHz |  |  | QPSK | 75 | 75 |
| 20MHz |  |  | QPSK | 100 | 100 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.3.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.1.4.3

##### 6.6.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.6.1.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously power control "up" commands to the UE until the UE transmits at PUMAX level.

3.	Measure the power spectrum distribution within two times or more range over the requirement for Occupied Bandwidth specification centring on the current carrier frequency. The characteristic of the filter shall be approximately Gaussian (typical spectrum analyzer filter). Other methods to measure the power spectrum distribution are allowed. The measuring duration is one active uplink subframe. For TDD slots with transient periods are not under test.

4.	Calculate the total power within the range of all frequencies measured in '3)' and save this value as "Total Power".

5.	Sum up the power upward from the lower boundary of the measured frequency range in '3)' and seek the limit frequency point by which this sum becomes 0,5 % of "Total Power" and save this point as "Lower Frequency".

6.	Sum up the power downward from the upper boundary of the measured frequency range in '3)' and seek the limit frequency point by which this sum becomes 0,5 % of "Total Power" and save this point as "Upper Frequency".

7.	Calculate the difference ("Upper Frequency"  "Lower Frequency" = "Occupied Bandwidth") between two limit frequencies obtained in '5)' and '6)'.

##### 6.6.1.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6

#### 6.6.1.5	Test requirement

The measured Occupied Bandwidth shall not exceed values in Table 6.6.1.5-1.

Table 6.6.1.5-1: Occupied channel bandwidth

|  | Occupied channel bandwidth / channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Channel bandwidth [MHz] | 1.4 | 3 | 5 | 10 | 15 | 20 |

### 6.6.1A	Occupied bandwidth for CA

#### 6.6.1A.0	Minimum conformance requirements

For inter-band carrier aggregation with uplink assigned to two E-UTRA bands the occupied bandwidth is defined per component carrier. Occupied bandwidth is the bandwidth containing 99 % of the total integrated mean power of the transmitted spectrum on assigned channel bandwidth on the component carrier. The occupied bandwidth shall be less than the channel bandwidth specified in Table 6.6.1.3-1.

For intra-band contiguous carrier aggregation the occupied bandwidth is a measure of the bandwidth containing 99 % of the total integrated power of the transmitted spectrum. The OBW shall be less than the aggregated channel bandwidth defined in subclause 5.4.2A.

For intra-band non-contiguous carrier aggregation sub-block occupied bandwidth is defined as the bandwidth containing 99 % of the total integrated mean power of the transmitted spectrum on the sub-block. In case the sub-block consists of one component carrier the occupied bandwidth of the sub-block shall be less than the channel bandwidth specified in Table 6.6.1.3-1.

The normative reference for this requirement is TS 36.101[2] clause 6.6.1A.

#### 6.6.1A.1	Occupied bandwidth for CA (intra-band contiguous DL CA and UL CA)

##### 6.6.1A.1.1	Test purpose

To verify that the UE occupied bandwidth for intra-band contiguous CA for all transmission bandwidth configurations supported by the UE are less than their specific limits

##### 6.6.1A.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 10 and forward that support intra-band contiguous DL CA and UL CA.

##### 6.6.1A.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.1A.0.

##### 6.6.1A.1.4	Test description

6.6.1A.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on E-UTRA CA configurations specified in table 5.4.2A.1-1. All of these configurations shall be tested with applicable test parameters for each CA configuration, and are shown in table 6.6.1A.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.1A.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  | Mid range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  | All |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 25 | 25 |  | QPSK | 50 | P_25@0 | P_25@0 | - | - |
| 25 | 50 |  | QPSK | 75 | P_25@0 | P_50@0 |  |  |
| 25 | 75 |  | QPSK | 100 | P_25@0 | P_75@0 | - | - |
| 25 | 100 | N/A for this test | QPSK | 125 | P_25@0 | S_100@0 | - | - |
| 50 | 50 |  | QPSK | 100 | P_50@0 | P_50@0 |  |  |
| 50 | 75 |  | QPSK | 125 | P_50@0 | S_75@0 | - | - |
| 50 | 100 |  | QPSK | 150 | P_50@0 | S_100@0 | - | - |
| 75 | 25 |  | QPSK | 100 | P_75@0 | S_25@0 | - | - |
| 75 | 50 |  | QPSK | 125 | P_75@0 | S_50@0 | - | - |
| 75 | 75 |  | QPSK | 150 | P_75@0 | S_75@0 | - | - |
| 75 | 100 |  | QPSK | 175 | P_75@0 | S_100@0 | - | - |
| 100 | 25 |  | QPSK | 125 | P_100@0 | S_25@0 | - | - |
| 100 | 50 |  | QPSK | 150 | P_100@0 | S_50@0 | - | - |
| 100 | 75 |  | QPSK | 175 | P_100@0 | S_75@0 | - | - |
| 100 | 100 |  | QPSK | 200 | P_100@0 | S_100@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.6.1A.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.1A.1.4.3.

6.6.1A.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.6.1A.1.4.3

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.6.1A.1.4.1-1 on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands to the UE until the UE transmits at PUMAX level.

6.	Measure the power spectrum distribution over all component carriers within two times or more range over the requirement for Occupied Bandwidth for CA specification centring on the current carrier frequency in CA configuration. The characteristic of the filter shall be approximately Gaussian (typical spectrum analyzer filter). Other methods to measure the power spectrum distribution are allowed. The measuring duration is one active uplink subframe. For TDD slots with transient periods are not under test.

7.	Calculate the total power for the total integrated power of the transmitted spectrum within the range of all frequencies measured in '6)' and save this values as "Total Power".

8.	Sum up the power upward from the lower boundary of the measured frequency range in '6)' and seek the limit frequency point by which this sum becomes 0,5 % of "Total Power" and save this point as "Lower Frequency".

9.	Sum up the power downward from the upper boundary of the measured frequency range in '6)' and seek the limit frequency point by which this sum becomes 0,5 % of "Total Power" and save this point as "Upper Frequency".

10.	Calculate the difference ("Upper Frequency"  "Lower Frequency" = "Occupied Bandwidth") between two limit frequencies obtained in '8)' and '9)'.

6.6.1A.1.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

##### 6.6.1A.1.5	Test Requirements

The measured Occupied Bandwidth shall not exceed values of aggregated channel bandwidth as defined in section 5.4.2A for intra-band contiguous carrier aggregation.

#### 6.6.1A.2	Occupied bandwidth for CA (inter-band DL CA and UL CA)

Editor’s note:	This test case is not complete for UL LAA. Following aspects are either missing or not yet determined:

-	UL RMC is missing in Annex A.

##### 6.6.1A.2.1	Test purpose

To verify that the UE occupied bandwidth for inter-band CA for all transmission bandwidth configurations supported by the UE are less than their specific limits

##### 6.6.1A.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 11 and forward that support inter-band CA and UL CA.

The requirements also apply to all types of E-UTRA UE release 14 and forward that support uplink LAA.

##### 6.6.1A.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.1A.0.

##### 6.6.1A.2.4	Test description

6.6.1A.2.4.1	Initial condition

Same as in clause 6.6.1A.1.4.1 with the following exceptions:

-	Instead of Table 5.4.2A.1-1 use Table 5.4.2A.1-2 for CA configurations with two bands or Table 5.4.2A.1-2a for CA configurations with three bands.

-	Instead of Table 6.6.1A.1.4.1-1  use Table 6.6.1A.2.4.1-1 or Table 6.6.1A.2.4.1-2.

-	Instead of clause 6.6.1A.1.4.3 use clause 6.6.1A.2.4.3.

Table 6.6.1A.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  | Mid range for PCC and SCC |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 6 | 25 | N/Afor this test | QPSK | 31 | P_6@0 | S_25@0 | - | - |
| 6 | 50 |  | QPSK | 56 | P_6@0 | S_50@0 | - | - |
| 15 | 25 |  | QPSK | 40 | P_15@0 | S_25@0 | - | - |
| 25 | 6 |  | QPSK |  | S_25@0 | P_6@0 |  |  |
| 25 | 25 |  | QPSK | 50 | P_25@0 | S_25@0 | - | - |
| 25 | 50 |  | QPSK | 75 | P_25@0 | S_50@0 | - | - |
| 25 | 75 |  | QPSK |  | S_25@0 | P_75@0 |  |  |
| 50 | 50 |  | QPSK | 100 | P_50@0 | S_50@0 | - | - |
| 50 | 100 |  | QPSK | 150 | P_50@0 | S_100@0 | - | - |
| 75 | 25 |  | QPSK | 100 | P_75@0 | S_25@0 | - | - |
| 75 | 75 |  | QPSK | 150 | P_75@0 | S_75@0 | - | - |
| 75 | 100 |  | QPSK | 175 | P_75@0 | S_100@0 | - | - |
| 100 | 50 |  | QPSK | 150 | P_100@0 | S_50@0 | - | - |
| 100 | 100 |  | QPSK | 200 | P_100@0 | S_100@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. |  |  |  |  |  |  |  |  |

Table 6.6.1A.2.4.1-2: Test Configuration Table for UL LAA

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  | Mid range for PCC and SCC |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 15 | 100 |  | QPSK | 115 | P_15@0 | S_100@0 | - | - |
| 25 | 100 |  | QPSK | 125 | P_25@0 | S_100@0 | - | - |
| 50 | 100 |  | QPSK | 150 | P_50@0 | S_100@0 | - | - |
| 100 | 100 |  | QPSK | 200 | P_100@0 | S_100@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each LAA CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. |  |  |  |  |  |  |  |  |

6.6.1A.2.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.6.1A.2.4.3

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 (or DCI format 0A for SCC in the case of uplink LAA) for C_RNTI to schedule the UL RMC according to Table 6.6.1A.2.4.1-1 or Table 6.6.1A.2.4.1-2 on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands to the UE until the UE transmits at PUMAX level.

6.	Measure the power spectrum distribution of both PCC and SCC within two times or more range over the requirement for Occupied Bandwidth specification centring on the current carrier frequencies respectively. The characteristic of the filter shall be approximately Gaussian (typical spectrum analyzer filter). Other methods to measure the power spectrum distribution are allowed. The measuring duration is one active uplink subframe. For TDD slots with transient periods are not under test.

7.	Calculate the total power for the total integrated power of the transmitted spectrum within the range of all frequencies measured in '6)' and save this values as "Total Power" for both PCC and SCC.

8.	Sum up the power upward from the lower boundary of the measured frequency range in '6)' and seek the limit frequency point by which this sum becomes 0,5 % of "Total Power" and save this point as "Lower Frequency" for both PCC and SCC.

9.	Sum up the power downward from the upper boundary of the measured frequency range in '3)' and seek the limit frequency point by which this sum becomes 0,5 % of "Total Power" and save this point as "Upper Frequency" for both PCC and SCC.

10.	Calculate the difference ("Upper Frequency"  "Lower Frequency" = "Occupied Bandwidth") between two limit frequencies obtained in '8)' and '9)' for both PCC and SCC.

6.6.1A.2.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

##### 6.6.1A.2.5	Test Requirements

The measured Occupied Bandwidth for each component carrier shall not exceed values in Table 6.6.1A.2.5-1.

Table 6.6.1A.2.5-1: Occupied channel bandwidth

|  | Occupied channel bandwidth / channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Channel bandwidth [MHz] | 1.4 | 3 | 5 | 10 | 15 | 20 |

#### 6.6.1A.3	Occupied bandwidth for CA (intra-band non-contiguous DL CA and UL CA)

##### 6.6.1A.3.1	Test purpose

To verify that the UE occupied bandwidth for intra-band non-contiguous CA for all transmission bandwidth configurations supported by the UE are less than their specific limits

##### 6.6.1A.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 11 and forward that support intra-band non-contiguous DL CA and UL CA.

##### 6.6.1A.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.1A.0.

##### 6.6.1A.3.4	Test description

6.6.1A.3.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on E-UTRA CA configurations specified in table 5.4.2A.1-3. All of these configurations shall be tested with applicable test parameters for each CA configuration, and are shown in table 6.6.1A.3.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.1A.3.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Refer to test points A: Low Wgap, Maximum Wgap |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  | Refer to test points All PCC NRB = SCC NRB test points |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  | Wgap [MHz] | DL Allocation |  | UL Allocation |  |  |  |
|  | PCC NRB | SCCs NRB |  | PCC & SCC RB allocation |  | CC MOD | PCC NRB_alloc (LCRB @ RBstart) | CC MOD | SCC NRB_alloc (LCRB @ RBstart) |
| Test Parameters for CA_4A-4A Configurations |  |  |  |  |  |  |  |  |  |
| 1 | 25 | 25 | 5 | NA |  | QPSK | P_25@0 | QPSK | S_25@0 |
| 2 | 50 | 50 | 25 |  |  | QPSK | P_50@0 | QPSK | S_50@0 |
| 3 | 75 | 75 | 15 |  |  | QPSK | P_75@0 | QPSK | S_100@0 |
| 4 | 100 | 100 | 5 |  |  | QPSK | P_100@0 | QPSK | S_100@0 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-3. Note 2: The carrier centre frequency of PCC in the UL operating band is configured closer to the DL operating band. |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.6.1A.3.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.1A.3.4.3.

6.6.1A.3.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels except PHICH.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.6.1A.3.4.3

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.6.1A.4.1-1 on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands to the UE until the UE transmits at PUMAX level.

6.	Measure the power spectrum distribution of each sub-block over all component carriers within the requirement for Occupied Bandwidth for CA specification + 5MHz, centring on the carrier frequency of each sub-block in CA configuration. The characteristic of the filter shall be approximately Gaussian (typical spectrum analyzer filter). Other methods to measure the power spectrum distribution are allowed. The measuring duration is one active uplink subframe. For TDD slots with transient periods are not under test.

7.	Calculate the total power for the total integrated power of the transmitted spectrum within the range of all frequencies measured for each sub-block in '6)' and save this values as "Total Power".

8.	Sum up the power upward from the lower boundary of the measured frequency range in '3)' and seek the limit frequency point by which this sum becomes 0,5 % of "Total Power" and save this point as "Lower Frequency".

9.	Sum up the power downward from the upper boundary of the measured frequency range in '3)' and seek the limit frequency point by which this sum becomes 0,5 % of "Total Power" and save this point as "Upper Frequency".

10.	Calculate the difference ("Upper Frequency"  "Lower Frequency" = "Occupied Bandwidth") between two limit frequencies obtained in '8)' and '9')' for each sub-block.

6.6.1A.3.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

##### 6.6.1A.3.5	Test Requirements

The measured Occupied Bandwidth shall not exceed values of channel bandwidth as defined in section 5.4.2A for intra-band non-contiguous carrier aggregation.

#### 6.6.1A.4	Occupied bandwidth for CA (3DL CA and 3UL CA)

Editor’s Note: This test case is incomplete. The following items are missing or are incomplete:

-The test applicability needs to be added to TS36.521-2

##### 6.6.1A.4.1	Test purpose

To verify that the UE occupied bandwidth for CA in all transmission bandwidth configurations supported by the UE are less than their specific limits

##### 6.6.1A.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 13 and forward that support 3DL CA and 3UL CA.

##### 6.6.1A.4.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.1A.0.

##### 6.6.1A.4.4	Test description

6.6.1A.4.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on E UTRA CA configurations specified in table 5.4.2A.1-1, 5.4.2A.1-2 and 5.4.2A.1-2a. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 6.5.1A.1.4.1-1 for interband and combinations of interband and intraband and in table 6.5.1A.1.4.1-2 for intraband contiguous only. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.1A.4.4.1-1: Test Configuration Table for interband and combinations of interband and intraband

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  | Mid range for PCC and SCC |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  | Lowest NRB for PCC and SCCs Highest NRB for PCC and SCCs |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 6 | 25 |  | QPSK | 56 | P_6@0 | S_25@0 | S_25@0 | - |
| 6 | 50 | N/A for this test | QPSK | 106 | P_6@0 | S_50@0 | S_50@0 | - |
| 15 | 25 |  | QPSK | 65 | P_15@0 | S_25@0 | S_25@0 | - |
| 25 | 25 |  | QPSK | 75 | P_25@0 | S_25@0 | S_25@0 | - |
| 25 | 50 |  | QPSK | 125 | P_25@0 | S_50@0 | S_50@0 | - |
| 50 | 50 |  | QPSK | 150 | P_50@0 | S_50@0 | S_50@0 | - |
| 50 | 100 |  | QPSK | 250 | P_50@0 | S_100@0 | S_100@0 | - |
| 75 | 75 |  | QPSK | 225 | P_75@0 | S_75@0 | S_75@0 | - |
| 75 | 100 |  | QPSK | 275 | P_75@0 | S_100@0 | S_100@0 | - |
| 100 | 50 |  | QPSK | 200 | P_100@0 | S_50@0 | S_50@0 | - |
| 100 | 100 |  | QPSK | 300 | P_100@0 | S_100@0 | S_100@0 | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1, 5.4.2A.1-2 and 5.4.2A.1-2a. |  |  |  |  |  |  |  |  |

Table 6.6.1A.4.4.1-2: Test Configuration Table for intraband contiguous only

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  | Mid range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  | All |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 25 | 50 |  | QPSK | 125 | P_25@0 | S_50@0 | S_50@0 |  |
| 25 | 100 | N/A for this test | QPSK | 225 | P_25@0 | S_100@0 | S_100@0 | - |
| 50 | 50 |  | QPSK | 150 | P_50@0 | S_50@0 | S_50@0 |  |
| 50 | 75 |  | QPSK | 200 | P_50@0 | S_75@0 | S_75@0 | - |
| 50 | 100 |  | QPSK | 250 | P_50@0 | S_100@0 | S_100@0 | - |
| 75 | 50 |  | QPSK | 175 | P_75@0 | S_50@0 | S_50@0 | - |
| 75 | 75 |  | QPSK | 225 | P_75@0 | S_75@0 | S_75@0 | - |
| 75 | 100 |  | QPSK | 275 | P_75@0 | S_100@0 | S_100@0 | - |
| 100 | 25 |  | QPSK | 150 | P_100@0 | S_25@0 | S_25@0 | - |
| 100 | 50 |  | QPSK | 200 | P_100@0 | S_50@0 | S_50@0 | - |
| 100 | 75 |  | QPSK | 250 | P_100@0 | S_75@0 | S_75@0 | - |
| 100 | 100 |  | QPSK | 300 | P_100@0 | S_100@0 | S_100@0 | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.6.1A.4.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.1A.4.1.4.3.

6.6.1A.4.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.6.1A.4.1.4.3

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.6.1A.4.1.4.1-1 on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands to the UE until the UE transmits at PUMAX level.

6.	Measure the power spectrum distribution over all component carriers within two times or more range over the requirement for Occupied Bandwidth for CA specification centring on the current carrier frequency in CA configuration. The characteristic of the filter shall be approximately Gaussian (typical spectrum analyzer filter). Other methods to measure the power spectrum distribution are allowed. The measuring duration is one active uplink subframe. For TDD slots with transient periods are not under test.

7.	Calculate the total power for the total integrated power of the transmitted spectrum within the range of all frequencies measured in '6)' and save this value as "Total Power".

8.	Sum up the power upward from the lower boundary of the measured frequency range in '6)' and seek the limit frequency point by which this sum becomes 0,5 % of "Total Power" and save this point as "Lower Frequency".

9.	Sum up the power downward from the upper boundary of the measured frequency range in '6)' and seek the limit frequency point by which this sum becomes 0,5 % of "Total Power" and save this point as "Upper Frequency".

10.	Calculate the difference ("Upper Frequency"  "Lower Frequency" = "Occupied Bandwidth") between two limit frequencies obtained in '8)' and '9)'.

6.6.1A.4.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

##### 6.6.1A.4.5	Test Requirements

The measured Occupied Bandwidth shall not exceed values of aggregated channel bandwidth as defined in section 5.4.2A.

#### 6.6.1A.5	Occupied bandwidth for CA (4UL CA)

##### 6.6.1A.5.1	Test purpose

To verify that the UE occupied bandwidth for CA in all transmission bandwidth configurations supported by the UE are less than their specific limits

##### 6.6.1A.5.2	Test applicability

This test case applies to all types of E-UTRA UE release 11 and forward that support 4DL CA and 4UL CA.

##### 6.6.1A.5.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.1A.0.

##### 6.6.1A.5.4	Test description

6.6.1A.5.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on E UTRA CA configurations specified in table 5.4.2A.1. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 6.6.1A.5.4.1-1 to 6.6.1A.5.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.1A.5.4.1-1: Test Configuration Table for intra-band contiguous CA

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  | Mid range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  | All |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCCs RB allocation |  | NRB_alloc | PCC & SCC1 & SCC2 & SCC3 RB allocations (LCRB @ RBstart) |  |  |  |
| 25 | 50 | N/A for this test | QPSK | 175 | P_25@0 | S_50@0 | S_50@0 | S_50@0 |
| 25 | 100 |  | QPSK | 325 | P_25@0 | S_100@0 | S_100@0 | S_100@0 |
| 50 | 50 |  | QPSK | 200 | P_50@0 | S_50@0 | S_50@0 | S_50@0 |
| 50 | 75 |  | QPSK | 275 | P_50@0 | S_75@0 | S_75@0 | S_75@0 |
| 50 | 100 |  | QPSK | 350 | P_50@0 | S_100@0 | S_100@0 | S_100@0 |
| 75 | 50 |  | QPSK | 225 | P_75@0 | S_50@0 | S_50@0 | S_50@0 |
| 75 | 75 |  | QPSK | 300 | P_75@0 | S_75@0 | S_75@0 | S_75@0 |
| 75 | 100 |  | QPSK | 375 | P_75@0 | S_100@0 | S_100@0 | S_100@0 |
| 100 | 25 |  | QPSK | 175 | P_100@0 | S_25@0 | S_25@0 | S_25@0 |
| 100 | 50 |  | QPSK | 250 | P_100@0 | S_50@0 | S_50@0 | S_50@0 |
| 100 | 75 |  | QPSK | 325 | P_100@0 | S_75@0 | S_75@0 | S_75@0 |
| 100 | 100 |  | QPSK | 400 | P_100@0 | S_100@0 | S_100@0 | S_100@0 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. |  |  |  |  |  |  |  |  |

Table 6.6.1A.5.4.1-2: Test Configuration Table for inter-band CA

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  | Mid range for PCC and SCC |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  | Lowest NRB for PCC and SCCs Highest NRB for PCC and SCCs |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCCs RB allocation |  | NRB_alloc | PCC & SCC1 & SCC2 & SCC3 RB allocations (LCRB @ RBstart) |  |  |  |
| 6 | 25 |  | QPSK | 81 | P_6@0 | S_25@0 | S_25@0 | S_25@0 |
| 6 | 50 | N/A for this test | QPSK | 156 | P_6@0 | S_50@0 | S_50@0 | S_50@0 |
| 15 | 25 |  | QPSK | 90 | P_15@0 | S_25@0 | S_25@0 | S_25@0 |
| 25 | 6 |  | QPSK | 43 | P_25@0 | S_6@0 | S_6@0 | S_6@0 |
| 25 | 25 |  | QPSK | 100 | P_25@0 | S_25@0 | S_25@0 | S_25@0 |
| 25 | 50 |  | QPSK | 175 | P_25@0 | S_50@0 | S_50@0 | S_50@0 |
| 25 | 75 |  | QPSK | 250 | P_25@0 | S_75@0 | S_75@0 | S_75@0 |
| 50 | 50 |  | QPSK | 200 | P_50@0 | S_50@0 | S_50@0 | S_50@0 |
| 50 | 100 |  | QPSK | 350 | P_50@0 | S_100@0 | S_100@0 | S_100@0 |
| 75 | 25 |  | QPSK | 150 | P_75@0 | S_25@0 | S_25@0 | S_25@0 |
| 75 | 75 |  | QPSK | 300 | P_75@0 | S_75@0 | S_75@0 | S_75@0 |
| 75 | 100 |  | QPSK | 375 | P_75@0 | S_100@0 | S_100@0 | S_100@0 |
| 100 | 50 |  | QPSK | 250 | P_100@0 | S_50@0 | S_50@0 | S_50@0 |
| 100 | 100 |  | QPSK | 400 | P_100@0 | S_100@0 | S_100@0 | S_100@0 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.6.1A.5.4.1-1 to 6.6.1A.5.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.1A.5.1.4.3.

6.6.1A.5.4.2	Test procedure

1.	Configure SCCs according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCCs as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.6.1A.5.1.4.3

3.	SS activates SCCs by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.6.1A.5.1.4.1-1 to 6.6.1A.5.4.1-2 on both PCC and SCCs. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands to the UE until the UE transmits at PUMAX level.

6.	Measure the power spectrum distribution over all component carriers within two times or more range over the requirement for Occupied Bandwidth for CA specification centring on the current carrier frequency in CA configuration. The characteristic of the filter shall be approximately Gaussian (typical spectrum analyzer filter). Other methods to measure the power spectrum distribution are allowed. The measuring duration is one active uplink subframe. For TDD slots with transient periods are not under test.

7.	Calculate the total power for the total integrated power of the transmitted spectrum within the range of all frequencies measured in '6)' and save this value as "Total Power".

8.	Sum up the power upward from the lower boundary of the measured frequency range in '6)' and seek the limit frequency point by which this sum becomes 0,5 % of "Total Power" and save this point as "Lower Frequency".

9.	Sum up the power downward from the upper boundary of the measured frequency range in '6)' and seek the limit frequency point by which this sum becomes 0,5 % of "Total Power" and save this point as "Upper Frequency".

10.	Calculate the difference ("Upper Frequency"  "Lower Frequency" = "Occupied Bandwidth") between two limit frequencies obtained in '8)' and '9)'.

6.6.1A.5.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCCs configuration there are no additional message contents.

##### 6.6.1A.5.5	Test Requirements

The measured Occupied Bandwidth shall not exceed values of aggregated channel bandwidth as defined in section 5.4.2A.

### 6.6.1B	Occupied bandwidth for UL-MIMO

#### 6.6.1B.1	Test purpose

To verify that the UE occupied bandwidth for all transmission bandwidth configurations supported by each of transmit antenna of the UE are less than their specific limits.

#### 6.6.1B.2	Test applicability

This test applies to all types of E-UTRA UE release 10 and forward that support UL-MIMO.

#### 6.6.1B.3	Minimum conformance requirements

For UE with multiple transmit antenna connectors, the requirements for occupied bandwidth is specified at each transmit antenna connector. The occupied bandwidth is defined as the bandwidth containing 99 % of the total integrated mean power of the transmitted spectrum on the assigned channel at each transmit antenna connector.

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the occupied bandwidth at each transmitter antenna shall be less than the channel bandwidth specified in Table 6.6.1B.3-1 with the UL-MIMO configurations specified in Table 6.2.2B.3-2.

Table 6.6.1B.3-1: Occupied channel bandwidth

|  | Occupied channel bandwidth / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Channel bandwidth (MHz) | 1.4 | 3 | 5 | 10 | 15 | 20 |

The normative reference for this requirement is TS 36.101 [2] clause 6.6.1B.

#### 6.6.1B.4	Test description

##### 6.6.1B.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.1B.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.1B.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  | Normal |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 |  | Mid range |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  | All |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
|  | Downlink Configuration |  | Uplink Configuration |  |  |
| Ch BW | N/A for Occupied bandwidth |  | Mod'n | RB allocation |  |
|  |  |  |  | FDD | TDD |
| 1.4MHz |  |  | QPSK | 6 | 6 |
| 3MHz |  |  | QPSK | 15 | 15 |
| 5MHz |  |  | QPSK | 25 | 25 |
| 10MHz |  |  | QPSK | 50 | 50 |
| 15MHz |  |  | QPSK | 75 | 75 |
| 20MHz |  |  | QPSK | 100 | 100 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.28.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.1B.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.1B.4.3.

##### 6.6.1B.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 4 for C_RNTI to schedule the UL RMC according to Table 6.6.1B.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously power control "up" commands to the UE until the UE transmits at PUMAX level.

3.	Measure the power spectrum distribution within two times or more range over the requirement for Occupied Bandwidth specification centring on the current carrier frequency. The characteristic of the filter shall be approximately Gaussian (typical spectrum analyzer filter). Other methods to measure the power spectrum distribution are allowed. The measuring duration is one active uplink subframe. For TDD slots with transient periods are not under test.

4.	Calculate the total power within the range of all frequencies measured in '3)' and save this value as "Total Power".

5.	Sum up the power upward from the lower boundary of the measured frequency range in '3)' and seek the limit frequency point by which this sum becomes 0,5 % of "Total Power" and save this point as "Lower Frequency".

6.	Sum up the power downward from the upper boundary of the measured frequency range in '3)' and seek the limit frequency point by which this sum becomes 0,5 % of "Total Power" and save this point as "Upper Frequency".

7.	Calculate the difference ("Upper Frequency"  "Lower Frequency" = "Occupied Bandwidth") between two limit frequencies obtained in '5)' and '6)'.

8.	Repeat step 3) until 7) for each of transmit antenna of the UE.

##### 6.6.1B.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6.6.6.1B.5	Test requirement

The measured Occupied Bandwidth at each transmit antenna of UE shall not exceed values in Table 6.6.1B.5-1.

Table 6.6.1B.5-1: Occupied channel bandwidth

|  | Occupied channel bandwidth / channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Channel bandwidth [MHz] | 1.4 | 3 | 5 | 10 | 15 | 20 |

### 6.6.1E	Occupied bandwidth for UE category 0

#### 6.6.1E.1	Test purpose

Same test purpose as in clause 6.6.1.1.

#### 6.6.1E.2	Test applicability

This test case applies to all types of E-UTRA UE release 12 and forward of UE category 0.

#### 6.6.1E.2	Minimum conformance requirements

Occupied bandwidth is a measure of the bandwidth containing 99 % of the total integrated mean power of the transmitted spectrum on the assigned channel. The occupied channel bandwidth for all transmission bandwidth configurations (Resources Blocks) should be less than the channel bandwidth specified in Table 6.6.1E.2-1

Table 6.6.1E.2-1: Occupied channel bandwidth

|  | Occupied channel bandwidth / channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Channel bandwidth [MHz] | 1.4 | 3 | 5 | 10 | 15 | 20 |

The normative reference for this requirement is TS 36.101 [2] clause 6.6.1.

#### 6.6.1E.4	Test description

##### 6.6.1E.4.1	Initial conditions

Same initial conditions as in clause 6.6.1.4.1 with the following exceptions:

-	Instead of Table 6.6.1.4.1-1  use Table 6.6.1E.4.1-1.

-	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.3 using only main UE Tx/Rx antenna.

Table 6.6.1E.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  | Normal |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 |  | Mid range |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  | All |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
|  | Downlink Configuration |  | Uplink Configuration |  |  |
| Ch BW | N/A for Occupied bandwidth |  | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | TDD |
| 1.4MHz |  |  | QPSK | 6 | 6 |
| 3MHz |  |  | QPSK | 15 | 15 |
| 5MHz |  |  | QPSK | 25 | 25 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. |  |  |  |  |  |

##### 6.6.1E.4.2	Test procedure

Same test procedure as in clause 6.6.1.4.2 with the following exception for HD-FDD:

-	In step 3, slots with transient periods are not under test. Half-duplex guard subframes are not under test.

##### 6.6.1E.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6

#### 6.6.1E.5	Test requirement

The measured Occupied Bandwidth shall not exceed values in Table 6.6.1E.5-1.

Table 6.6.1E.5-1: Occupied channel bandwidth

|  | Occupied channel bandwidth / channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Channel bandwidth [MHz] | 1.4 | 3 | 5 | NA | NA | NA |

### 6.6.1EA	Occupied bandwidth for UE category M1

#### 6.6.1EA.1	Test purpose

Same test purpose as in clause 6.6.1.1.

#### 6.6.1EA.2	Test applicability

This test case applies to all types of E-UTRA UE release 13 and forward of UE category M1.

#### 6.6.1EA.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.6.1.3.

#### 6.6.1EA.4	Test description

##### 6.6.1E.4A.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in clause 5.2E. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.1EA.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and MPDCCH before measurement are specified in Annex C.2.

Table 6.6.1EA.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 |  | Mid range |  |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  | 5 MHz |  |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  | Downlink Configuration |  | Uplink Configuration |  |  |  |
| Ch BW | N/A for Occupied bandwidth |  | Mod'n | RB allocation |  |  |
|  |  |  |  | FDD and HD-FDD | TDD | Narrowband index1 |
| 5MHz |  |  | QPSK | 6 | 6 | 0 |
| Note 1: Denote where in the channel bandwidth the narrowband shall be placed. Narrowband and narrowband index are defined in TS 36.211, 5.2.4. |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.1EA.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [7] clause 5.2A.2AA. Message contents are defined in clause 6.6.1EA.4.3.

##### 6.6.1EA.4.2	Test procedure

1.	The SS sends uplink scheduling information for each UL HARQ process via MPDCCH DCI format 6-0A for C_RNTI to schedule the UL RMC according to Table 6.6.1EA.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously power control "up" commands to the UE until the UE transmits at PUMAX level.

3.	Measure the power spectrum distribution within two times or more range over the requirement for Occupied Bandwidth specification centring on the current carrier frequency. The characteristic of the filter shall be approximately Gaussian (typical spectrum analyzer filter). Other methods to measure the power spectrum distribution are allowed. The measuring duration is one active uplink subframe. For TDD slots with transient periods are not under test For HD-FDD slots with transient periods and Half-duplex guard subframe are not under test.

4.	Calculate the total power within the range of all frequencies measured in '3)' and save this value as "Total Power".

5.	Sum up the power upward from the lower boundary of the measured frequency range in '3)' and seek the limit frequency point by which this sum becomes 0,5 % of "Total Power" and save this point as "Lower Frequency".

6.	Sum up the power downward from the upper boundary of the measured frequency range in '3)' and seek the limit frequency point by which this sum becomes 0,5 % of "Total Power" and save this point as "Upper Frequency".

7.	Calculate the difference ("Upper Frequency"  "Lower Frequency" = "Occupied Bandwidth") between two limit frequencies obtained in '5)' and '6)'.

##### 6.6.1EA.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the condition CEModeA.

#### 6.6.1EA.5	Test requirement

The measured Occupied Bandwidth shall not exceed values in Table 6.6.1EA.5-1.

Table 6.6.1EA.5-1: Occupied channel bandwidth

|  | Occupied channel bandwidth / channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Channel bandwidth [MHz] | N/A | N/A | 1.4 | N/A | N/A | N/A |

### 6.6.1EB	Occupied bandwidth for UE category 1bis

#### 6.6.1EB.1	Test purpose

Same test purpose as in clause 6.6.1.1.

#### 6.6.1EB.2	Test applicability

This test case applies to all types of E-UTRA UE release 13 and forward of UE category 1bis.

#### 6.6.1EB.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.6.1.3.

#### 6.6.1EB.4	Test description

##### 6.6.1EB.4.1	Initial conditions

Same initial conditions as in clause 6.6.1.4.1 with the following exceptions:

-	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.3 using only main UE Tx/Rx antenna.

##### 6.6.1EB.4.2	Test procedure

Same test procedure as in clause 6.6.1.4.2

##### 6.6.1EB.4.3	Message contents

Same message contents as in clause 6.6.1.4.3

#### 6.6.1EB.5	Test requirement

The measured Occupied Bandwidth shall not exceed values in Table 6.6.1EB.5-1.

Table 6.6.1E.5-1: Occupied channel bandwidth

|  | Occupied channel bandwidth / channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Channel bandwidth [MHz] | 1.4 | 3 | 5 | 10 | 15 | 20 |

### 6.6.1EC	Occupied bandwidth for UE category M2

#### 6.6.1EC.1	Test purpose

Same test purpose as in clause 6.6.1.1.

#### 6.6.1EC.2	Test applicability

This test case applies to all types of E-UTRA UE release 14 and forward of UE category M2.

#### 6.6.1EC.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.6.1.3.

#### 6.6.1EC.4	Test description

##### 6.6.1EC.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in clause 5.2E. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.1EC.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and MPDCCH before measurement are specified in Annex C.2.

Table 6.6.1EC.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 |  | Mid range |  |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  | 1.4MHz, 3MHz, 5MHz, 10MHz |  |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  | Downlink Configuration |  | Uplink Configuration |  |  |  |
| Ch BW | N/A for Occupied bandwidth |  | Mod'n | RB allocation |  |  |
|  |  |  |  | FDD and HD-FDD | TDD | Narrowband index1 |
| 1.4MHz |  |  | QPSK | 6 | 6 | 0 |
| 3MHz |  |  | QPSK | 12 | 12 | 0 |
| 5MHz |  |  | QPSK | 24 | 24 | 0 |
| 10MHz (Note 2) |  |  | QPSK | 24 | - | 0 |
| Note 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.4. Note 2: Channel bandwidths 10MHz is tested only for E-UTRA operating band 13. |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.1EC.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [7] clause 5.2A.2AA. Message contents are defined in clause 6.6.1EC.4.3.

##### 6.6.1EC.4.2	Test procedure

1.	The SS sends uplink scheduling information for each UL HARQ process via MPDCCH DCI format 6-0A for C_RNTI to schedule the UL RMC according to Table 6.6.1EC.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously power control "up" commands to the UE until the UE transmits at PUMAX level.

3.	Measure the power spectrum distribution within two times or more range over the requirement for Occupied Bandwidth specification centring on the current carrier frequency. The characteristic of the filter shall be approximately Gaussian (typical spectrum analyzer filter). Other methods to measure the power spectrum distribution are allowed. The measuring duration is one active uplink subframe. For TDD slots with transient periods are not under test For HD-FDD slots with transient periods and Half-duplex guard subframe are not under test.

4.	Calculate the total power within the range of all frequencies measured in '3)' and save this value as "Total Power".

5.	Sum up the power upward from the lower boundary of the measured frequency range in '3)' and seek the limit frequency point by which this sum becomes 0,5 % of "Total Power" and save this point as "Lower Frequency".

6.	Sum up the power downward from the upper boundary of the measured frequency range in '3)' and seek the limit frequency point by which this sum becomes 0,5 % of "Total Power" and save this point as "Upper Frequency".

7.	Calculate the difference ("Upper Frequency"  "Lower Frequency" = "Occupied Bandwidth") between two limit frequencies obtained in '5)' and '6)'.

##### 6.6.1EC.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the condition CEModeA.

#### 6.6.1EC.5	Test requirement

The measured Occupied Bandwidth shall not exceed values in Table 6.6.1EC.5-1.

Table 6.6.1EC.5-1: Occupied channel bandwidth

|  | Occupied channel bandwidth / channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Channel bandwidth [MHz] | 1.4 | 3.0 | 5 | 5 | N/A | N/A |

### 6.6.1F	Occupied bandwidth for category NB1 and NB2

#### 6.6.1F.1	Test purpose

To verify that the UE occupied bandwidth for all transmission bandwidth configurations supported by the UE are less than their specific limits.

#### 6.6.1F.2	Test applicability

This test case applies to all types of UE release 13 and forward of category NB1.

This test case applies to all types of UE release 14 and forward of category NB2.

This test case applies to all types of E-UTRA TDD UE release 15 and forward of category NB1 and NB2.

#### 6.6.1F.3	Minimum conformance requirements

The occupied bandwidth is defined as the bandwidth containing 99 % of the total integrated mean power of the transmitted spectrum on the assigned channel at the transmit antenna connector. Occupied bandwidth shall be less than the channel bandwidth of category NB1 and NB2 that is 200 kHz.

The normative reference for this requirement is TS 36.101[2] clause 6.6.1F.

#### 6.6.1F.4	Test description

##### 6.6.1F.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions and test frequencies based on the subset of E-UTRA operating bands defined for NB-IoT in clause 5.2F. All of these configurations shall be tested with applicable test parameters shown in table 6.6.1F.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2.4. Configurations of NPDSCH and NPDCCH before measurement are specified in Annex C.2.

Table 6.6.1F.4.1-1: Test Configuration Table for FDD & TDD

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 8.1.1 |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 8.1.3.1 |  | Frequency ranges defined in Annex K.1.1 |  |  |
| Test Parameters |  |  |  |  |
| Configuration ID | Downlink Configuration | Uplink Configuration |  |  |
|  | N/A | Modulation | Ntones | Sub-carrier spacing (kHz) |
| 1 |  | QPSK | 1@0 | 3.75kHz |
| 2 |  | QPSK | 1@0 | 15kHz |
| 3 (Note 1) |  | QPSK | 12@0 | 15kHz |
| Note 1: Applicable to UE supporting UL multi-tone transmissions |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.3 using only main Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.4.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.1F.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5. Message contents are defined in clause 6.6.1F.4.3

##### 6.6.1F.4.2	Test procedure

1.	SS sends uplink scheduling information via NPDCCH DCI format N0 for C_RNTI to schedule the UL RMC according to Table 6.6.1F.4.1-1. Since the UE has no payload and no loopback data to send, the UE sends uplink MAC padding bits on the UL RMC. (UE should be already transmitting PUMAX after Initial Conditions setting).

2.	Measure the power spectrum distribution within two times or more range over the requirement for Occupied Bandwidth specification centring on the current carrier frequency. The characteristic of the filter shall be approximately Gaussian (typical spectrum analyzer filter). Other methods to measure the power spectrum distribution are allowed. The measurement duration is at least one sub-frame (1ms) for 15 KHz channel spacing, and at least a 2ms slot (excluding the 2304Ts gap when UE is not transmitting) respectively for the 3.75 KHz channel spacing. For TDD slots with transient periods are not under test.

3.	Calculate the total power within the range of all frequencies measured in '2)' and save this value as "Total Power".

4.	Sum up the power upward from the lower boundary of the measured frequency range in '2)' and seek the limit frequency point by which this sum becomes 0,5 % of "Total Power" and save this point as "Lower Frequency".

5.	Sum up the power downward from the upper boundary of the measured frequency range in '2)' and seek the limit frequency point by which this sum becomes 0,5 % of "Total Power" and save this point as "Upper Frequency".

6.	Calculate the difference ("Upper Frequency "  "Lower Frequency" = "Occupied Bandwidth") using the limit frequencies obtained in '4)' or '5)'.

NOTE 1:	For configuration IDs applicable to UE depending on UE capability in Test Configuration Table with different UL sub-carrier spacing, the SS shall release the connection through State 3A-NB and finally ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5 using the appropriate UL subcarrier spacing in Random Access Response message.

##### 6.6.1F.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 8.1.6.

#### 6.6.1F.5	Test requirement

The measured Occupied Bandwidth shall not exceed 200kHz.

### 6.6.1G	Occupied bandwidth for V2X Communication

#### 6.6.1G.1	Occupied bandwidth for V2X Communication / Non-concurrent with E-UTRA uplink transmission

##### 6.6.1G.1.1	Test purpose

Same test purpose as in clause 6.6.1.1.

##### 6.6.1G.1.2	Test applicability

This test case applies to all types of UE that support V2X Sidelink communication and Band 47.

##### 6.6.1G.1.3	Minimum conformance requirements

When UE is configured for E-UTRA V2X sidelink transmissions non-concurrent with E-UTRA uplink transmissions for E-UTRA V2X operating bands specified in TS 36.101 [2] Table 5.5G-1, the requirements in subclause 6.6.1 apply for E-UTRA V2X sidelink transmission.

The normative reference for this requirement is TS 36.101 [2] clause 6.6.1G.

##### 6.6.1G.1.4	Test description

###### 6.6.1G.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on E-UTRA operating bands specified in table 5.4.2G.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.1G.4.1-1. The details of the V2X reference measurement channels (RMCs) are specified in Annexes A.8.3 and the GNSS configuration in TS 36.508 [7] subclause 4.11.

Table 6.6.1G.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  | Normal |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  | Mid range |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  | All |  |
| Test Parameters for Channel Bandwidths |  |  |  |
|  | V2X Configuration to Transmit |  |  |
| Ch BW | Modulation |  | PSSCH RB allocation |
| 10MHz | QPSK |  | 48@2 |
| 20MHz | QPSK |  | 96@2 |

1.	Connect the SS to the UE antenna connectors and connect the GNSS simulator to the UE GNSS RX antenna connector as shown in TS 36.508 [7] Annex A, Figure A.92.

2.	The parameter settings for the V2X sidelink transmission over PC5 are pre-configured according to TS 36.508 [7] subclause 6.8.2. Message content exceptions are defined in clause 6.6.1G.1.4.3.

3.	The V2X reference measurement channel is set according to Table 6.6.1G.1.4.1-1.

3a.	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS36.508 [7] Table 4.11.2-2. Geographical area #1 is also pre-configured in the UE.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 5A-V2X in Transmit Mode according to TS 36.508 [7] clause 4.5.9.

6.	The GNSS simulator is triggered to start step 1 of Scenario #1 to simulate a location in the centre of Geographical area #1. Wait for the UE to acquire the GNSS signal and start to transmit.

###### 6.6.1G.1.4.2	Test procedure

1.	The UE starts to perform the V2X sidelink communication according to SL-V2X-Preconfiguration. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the V2X RMC.

2.	Measure the power spectrum distribution within two times or more range over the requirement for Occupied Bandwidth specification centring on the current carrier frequency. The characteristic of the filter shall be approximately Gaussian (typical spectrum analyzer filter). Other methods to measure the power spectrum distribution are allowed. The measuring duration is one active uplink subframe.

3.	Calculate the total power within the range of all frequencies measured in '2)' and save this value as "Total Power".

4.	Sum up the power upward from the lower boundary of the measured frequency range in '2)' and seek the limit frequency point by which this sum becomes 0,5 % of "Total Power" and save this point as "Lower Frequency".

5.	Sum up the power downward from the upper boundary of the measured frequency range in '2)' and seek the limit frequency point by which this sum becomes 0,5 % of "Total Power" and save this point as "Upper Frequency".

6.	Calculate the difference ("Upper Frequency"  "Lower Frequency" = "Occupied Bandwidth") between two limit frequencies obtained in '4)' and '5)'.

###### 6.6.1G.1.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6, 4.7.I and 4.10:

##### 6.6.1G.1.5	Test requirements

The measured Occupied Bandwidth shall not exceed values in Table 6.6.1G.1.5-1.

Table 6.6.1G.1.5-1: Occupied channel bandwidth for EUTRA band 47

|  | Occupied channel bandwidth / channel bandwidth |  |
| --- | --- | --- |
|  | 10 MHz | 20 MHz |
| Channel bandwidth [MHz] | 10 | 20 |

#### 6.6.1G.2	Occupied bandwidth for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions

##### 6.6.1G.2.1	Test purpose

Same test purpose as in clause 6.6.1.1.

##### 6.6.1G.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 14 and forward that support V2X communication.

##### 6.6.1G.2.3	Minimum conformance requirements

When UE is configured for simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions for inter-band E-UTRA V2X / E-UTRA bands specified in TS 36.101 [2] Table 5.5G-2, the requirements in subclause 6.6.1 apply for V2X sidelink transmission and the E-UTRA uplink transmission.

The normative reference for this requirement is TS 36.101 [2] clause 6.6.1G.

##### 6.6.1G.2.4	Test description

###### 6.6.1G.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in Table 5.4.2G.1-2. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.1G.2.4.1-1. The details of the uplink and V2X reference measurement channels (RMCs) are specified in Annexes A.2 and A.8.3 respectively. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.1G.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Mid range |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Lowest NRB_agg, Highest NRB_agg |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  | Downlink | Uplink Configuration |  | V2X Configuration |  |  |
| Ch BW | N/A for Occupied bandwidth | Mod'n | RB allocation | Ch BW | Mod'n | PSSCH RB allocation |
| 1.4 MHz |  | QPSK | 6 | 10 MHz | QPSK | 48@2 |
| 3 MHz |  | QPSK | 15 | 20 MHz | QPSK | 96@2 |
| 5 MHz |  | QPSK | 25 |  |  |  |
| 10 MHz |  | QPSK | 50 |  |  |  |
| 15 MHz |  | QPSK | 75 |  |  |  |
| 20 MHz |  | QPSK | 100 |  |  |  |
| Note 1: V2X con-current band configurations are checked separately for each band configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2G.1-2. |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.93a.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL and V2X Reference Measurement Channels are set according to Table 6.6.1G.2.4.1-1

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF-V2X according to TS 36.508 [7] clause 5.2A.2C. Message content exceptions are defined in clause 6.6.1G.2.4.3.

###### 6.6.1G.2.4.2	Test procedure

1.	The SS simultaneously sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC and V2X scheduling information via PDCCH DCI format 5A for SL_V_RNTI to schedule the V2X RMC according to Table 6.6.1G.2.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL and V2X RMCs.

2.	Send continuously power control "up" commands to the UE until the UE transmits at PUMAX level. Configure the UE to transmit PSCCH and PSSCH with the PUMAX level of each test points.

3.	Measure the power spectrum distribution of both concurrent bands within two times or more range over the requirement for Occupied Bandwidth specification centring on the current carrier frequency. The characteristic of the filter shall be approximately Gaussian (typical spectrum analyzer filter). Other methods to measure the power spectrum distribution are allowed. The measuring duration is one active uplink subframe. On E-UTRA carrier, the TDD slots with transient periods are not under test.

4.	Calculate the total power within the range of all frequencies measured in '3)' and save this value as "Total Power" for both concurrent bands.

5.	Sum up the power upward from the lower boundary of the measured frequency range in '3)' and seek the limit frequency point by which this sum becomes 0,5 % of "Total Power" and save this point as "Lower Frequency" for both concurrent bands.

6.	Sum up the power downward from the upper boundary of the measured frequency range in '3)' and seek the limit frequency point by which this sum becomes 0,5 % of "Total Power" and save this point as "Upper Frequency" for both concurrent bands.

7.	Calculate the difference ("Upper Frequency"  "Lower Frequency" = "Occupied Bandwidth") between two limit frequencies obtained in '5)' and '6)' for both concurrent bands.

###### 6.6.1G.2.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6.

##### 6.6.1G.2.5	Test requirements

The measured Occupied Bandwidth for both concurrent bands shall not exceed values in Table 6.6.1G.2.5-1.

Table 6.6.1G.2.5-1: Occupied channel bandwidth

|  | Occupied channel bandwidth / channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Channel bandwidth [MHz] | 1.4 | 3 | 5 | 10 | 15 | 20 |

#### 6.6.1G.3	Occupied bandwidth for V2X Communication / Intra-band contiguous multi-carrier operation

##### 6.6.1G.3.1	Test purpose

Same test purpose as in clause 6.6.1.1.

##### 6.6.1G.3.2	Test applicability

This test case applies to all types of UE that support Intra-band contiguous multi-carrier operation V2X Sidelink communication and Band 47.

##### 6.6.1G.3.3	Minimum conformance requirements

For intra-band contiguous multi-carrier operation, the occupied bandwidth is a measure of the bandwidth containing 99 % of the total integrated power of the transmitted spectrum. The OBW shall be less than the aggregated channel bandwidth defined in TS 36.101 [2] subclause 5.6A.

The normative reference for this requirement is TS 36.101 [2] clause 6.6.1G.

##### 6.6.1G.3.4	Test description

###### 6.6.1G.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in Table 5.4.2G.1-4. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.1G.3.4.1-1. The details of the V2X reference measurement channels (RMCs) are specified in Annexes A.8.3 and the GNSS configuration in TS 36.508 [7] subclause 4.11.

Table 6.6.1G.3.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Mid range |  |
| Test CC Combination setting(NRB_agg) as specified in subclause 5.4.2G |  |  | Lowest NRB_agg, Highest NRB_agg |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| V2X Multi-carrier Configuration |  | V2X PSSCH Allocation |  |  |
| CC1 NRB | CC2 NRB | CC1 & CC2 Mod |  | CC1 and CC2 RB Allocation |
| 50 | 50 | QPSK |  | 48@2+0@0 |
| 50 | 50 | QPSK |  | 48@2+48@2 |

1.	Connect the SS to the UE antenna connectors and connect the GNSS simulator to the UE GNSS RX antenna connector as shown in TS 36.508 [7] Annex A, in Figure A.89a.

2.	The parameter setting for the V2X sidelink transmission over PC5 is pre-configured according to TS 36.508[7] subclause 4.10.1. Message content exceptions are defined in clause 6.6.1G.3.4.3.

3.	The V2X Reference Measurement Channels are set according to Table 6.6.1G.3.4.1-1.

3a	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS36.508 [7] Table 4.11.2-2. Geographical area #1 is also pre-configured in the UE.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 5A-V2X according to TS 36.508 [7] clause 4.5.9.

6.	The GNSS simulator is triggered to start step 1 of Scenario #1 to simulate a location in the centre of Geographical area #1. Wait for the UE to acquire the GNSS signal and start to transmit.

###### 6.6.1G.3.4.2	Test procedure

1.	The UE starts to perform the V2X sidelink communication according to SL-V2X-Preconfiguration. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the V2X RMCs.

2.	Measure the power spectrum distribution within two times or more range over the requirement for Occupied Bandwidth for V2X multi-carrier configuration specification centring on the current carrier frequency. The characteristic of the filter shall be approximately Gaussian (typical spectrum analyzer filter). Other methods to measure the power spectrum distribution are allowed. The measuring duration is one active uplink subframe.

3.	Calculate the total power within the range of all frequencies measured in '2)' and save this value as "Total Power".

4.	Sum up the power upward from the lower boundary of the measured frequency range in '2)' and seek the limit frequency point by which this sum becomes 0,5 % of "Total Power" and save this point as "Lower Frequency".

5.	Sum up the power downward from the upper boundary of the measured frequency range in '2)' and seek the limit frequency point by which this sum becomes 0,5 % of "Total Power" and save this point as "Upper Frequency".

6.	Calculate the difference ("Upper Frequency"  "Lower Frequency" = "Occupied Bandwidth") between two limit frequencies obtained in '4)' and '5)'.

###### 6.6.1G.3.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6, 4.7.I and 4.10 with the following exceptions:

Table 6.6.1G.3.4.3-0: +CCUTLE (Initial condition)

Derivation Path: 36.508 Table 4.7I-2: +CCUTLE, condition Close and Transmit

##### 6.6.1G.3.5	Test requirements

The measured Occupied Bandwidth shall not exceed values of aggregated channel bandwidth as defined in section 5.4.2A for V2X intra-band multi-carrier configuration.

### 6.6.2	Out of band emission

Out of band emissions are unwanted emissions immediately outside the nominal channel resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions. This out of band emission limit is specified in terms of a Spectrum Emission Mask and Adjacent Channel Leakage power Ratio.

#### 6.6.2.1	Spectrum Emission Mask

##### 6.6.2.1.1	Test purpose

To verify that the power of any UE emission shall not exceed specified lever for the specified channel bandwidth.

##### 6.6.2.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 8 and forward.

##### 6.6.2.1.3	Minimum conformance requirements

The spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the edge of the assigned E-UTRA channel bandwidth. For frequencies greater than (ΔfOOB) as specified in Table 6.6.2.1.3-1 the spurious requirements in clause 6.6.3 are applicable.

The power of any UE emission shall not exceed the levels specified in Table 6.6.2.1.3-1 for the specified channel bandwidth.

Table 6.6.2.1.3-1: General E-UTRA spectrum emission mask

| Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -10 | -13 | -15 | -18 | -20 | -21 | 30 kHz |
| 1-2.5 | -10 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.5-2.8 | -25 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.8-5 |  | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 5-6 |  | -25 | -13 | -13 | -13 | -13 | 1 MHz |
| 6-10 |  |  | -25 | -13 | -13 | -13 | 1 MHz |
| 10-15 |  |  |  | -25 | -13 | -13 | 1 MHz |
| 15-20 |  |  |  |  | -25 | -13 | 1 MHz |
| 20-25 |  |  |  |  |  | -25 | 1 MHz |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

The normative reference for this requirement is TS 36.101 [2] clause 6.6.2.1.

##### 6.6.2.1.4	Test description

6.6.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2 respectively. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  | NC |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  | Lowest, 5MHz, 10MHz, Highest |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
|  | Downlink Configuration | Uplink Configuration |  |  |
| Ch BW | N/A for SEM testing | Mod'n | RB allocation |  |
|  |  |  | FDD | TDD |
| 1.4MHz |  | QPSK | 6 | 6 |
| 1.4MHz |  | QPSK | 5 | 5 |
| 1.4MHz |  | 16QAM | 5 | 5 |
| 1.4MHz |  | 16QAM | 6 | 6 |
| 3MHz |  | QPSK | 15 | 15 |
| 3MHz |  | QPSK | 4 | 4 |
| 3MHz |  | 16QAM | 4 | 4 |
| 3MHz |  | 16QAM | 15 | 15 |
| 5MHz |  | QPSK | 25 | 25 |
| 5MHz |  | QPSK | 8 | 8 |
| 5MHz |  | 16QAM | 8 | 8 |
| 5MHz |  | 16QAM | 25 | 25 |
| 10MHz |  | QPSK | 50 | 50 |
| 10MHz |  | QPSK | 12 | 12 |
| 10MHz |  | 16QAM | 12 | 12 |
| 10MHz |  | 16QAM | 50 (Note 4) | 50 (Note 4) |
| 15MHz |  | QPSK | 75 | 75 |
| 15MHz |  | QPSK | 16 | 16 |
| 15MHz |  | 16QAM | 16 | 16 |
| 15MHz |  | 16QAM | 75 (Note 4) | 75 (Note 4) |
| 20MHz |  | QPSK | 100 | 100 |
| 20MHz |  | QPSK | 18 | 18 |
| 20MHz |  | 16QAM | 18 | 18 |
| 20MHz |  | 16QAM | 100 (Note 4) | 100 (Note 4) |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The allowed MPR for maximum output power UE might apply is described in clause 6.2.3.3. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (max+1 - RB allocation) of the channel bandwidth. Note 4: Applies only for UE-Categories ≥2 |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure A.3.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.1.4.3.

6.6.2.1.4.2	Test procedure

SS sends uplink scheduling information via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.6.2.1.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Table 6.2.3.5-1 for Power Class 3 UEs or in 6.2.3_1.5-1 for HPUE (Power Class 1 or Power Class 2 UEs). The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

4.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to table 6.6.2.1.5-1 or 6.6.2.1.5-2 and using a rms detector, as applicable. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.6.2.1.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6.

##### 6.6.2.1.5	Test requirements

The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.3.5-1 as appropriate, and the power of any UE emission shall fulfil requirements in Table.6.6.2.1.5-1 or Table.6.6.2.1.5-2, as applicable.

Table 6.6.2.1.5-1: General E-UTRA spectrum emission mask, E-UTRA bands ≤ 3GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.5 | -11.5 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 1-2.5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 6-10 |  |  | -23.5 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.5 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.5 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

Table 6.6.2.1.5-2: General E-UTRA spectrum emission mask, 3GHz < E-UTRA bands ≤ 4.2GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.2 | -11.2 | -13.2 | -16.2 | -18.2 | -19.2 | 30 kHz |
| 1-2.5 | -8.2 | -8.2 | -8.2 | -8.2 | -8.2 | -8.2 | 1 MHz |
| 2.5-2.8 | -23.2 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.2 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 6-10 |  |  | -23.2 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.2 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.2 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.2 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

#### 6.6.2.1_1	Spectrum Emission Mask for Multi-Cluster PUSCH

##### 6.6.2.1_1.1	Test purpose

To verify that the power of any UE emission shall not exceed specified level for the specified channel bandwidth for Multi-Cluster PUSCH transmission.

##### 6.6.2.1_1.2	Test applicability

This test case applies to all types of E-UTRA UE release 10 and forward that support multi cluster PUSCH within a component carrier for the tested band.

##### 6.6.2.1_1.3	Minimum conformance requirements

The spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the edge of the assigned E-UTRA channel bandwidth. For frequencies greater than (ΔfOOB) as specified in Table 6.6.2.1_1.3-1 the spurious requirements in clause 6.6.3 are applicable.

The power of any UE emission shall not exceed the levels specified in Table 6.6.2.1_1.3-1 for the specified channel bandwidth.

Table 6.6.2.1_1.3-1: General E-UTRA spectrum emission mask

| Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -10 | -13 | -15 | -18 | -20 | -21 | 30 kHz |
| 1-2.5 | -10 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.5-2.8 | -25 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.8-5 |  | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 5-6 |  | -25 | -13 | -13 | -13 | -13 | 1 MHz |
| 6-10 |  |  | -25 | -13 | -13 | -13 | 1 MHz |
| 10-15 |  |  |  | -25 | -13 | -13 | 1 MHz |
| 15-20 |  |  |  |  | -25 | -13 | 1 MHz |
| 20-25 |  |  |  |  |  | -25 | 1 MHz |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

The normative reference for this requirement is TS 36.101 [2] clause 6.6.2.1.

##### 6.6.2.1_1.4	Test description

6.6.2.1_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.1_1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2 respectively. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.1_1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Highest |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
| Configuration ID | Downlink Configuration |  | Uplink Configuration |  |  |
|  | Ch BW | N/A for SEM testing | Mod'n | Cluster1 RB allocations (LCRB @ RBstart) | Cluster2 RB allocations (LCRB @ RBstart) |
| 1 | 5MHz |  | 16QAM | 2@0 | 1@24 |
| 2 | 5MHz |  | 16QAM | 18@0 | 2@22 |
| 3 | 5MHz |  | 16QAM | 2@0 | 18@6 |
| 4 | 10MHz |  | 16QAM | 3@0 | 2@48 |
| 5 | 10MHz |  | 16QAM | 42@0 | 3@45 |
| 6 | 10MHz |  | 16QAM | 3@0 | 42@6 |
| 7 | 15MHz |  | 16QAM | 8@0 | 7@68 |
| 8 | 15MHz |  | 16QAM | 60@0 | 4@68 |
| 9 | 15MHz |  | 16QAM | 4@0 | 60@12 |
| 10 | 20MHz |  | 16QAM | 4@0 | 4@96 |
| 11 | 20MHz |  | 16QAM | 92@0 | 4@96 |
| 12 | 20MHz |  | 16QAM | 4@0 | 92@8 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure A.3.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.1_1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.1.4.3.

6.6.2.1_1.4.2	Test procedure

1.	SS sends uplink scheduling information via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.6.2.1_1.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Table 6.2.3_2.5-1. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

4.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to table 6.6.2.1_1.5-1 or 6.6.2.1_1.5-2 and using a rms detector, as applicable. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.6.2.1_1.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6.

##### 6.6.2.1_1.5	Test requirements

The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.3_2.5-1 as appropriate, and the power of any UE emission shall fulfil requirements in Table 6.6.2.1_1.5-1 or Table 6.6.2.1_1.5-2, as applicable.

Table 6.6.2.1_1.5-1: General E-UTRA spectrum emission mask, E-UTRA bands ≤ 3GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.5 | -11.5 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 1-2.5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 6-10 |  |  | -23.5 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.5 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.5 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

Table 6.6.2.1_1.5-2: General E-UTRA spectrum emission mask, 3GHz < E-UTRA bands ≤ 4.2GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.2 | -11.2 | -13.2 | -16.2 | -18.2 | -19.2 | 30 kHz |
| 1-2.5 | -8.2 | -8.2 | -8.2 | -8.2 | -8.2 | -8.2 | 1 MHz |
| 2.5-2.8 | -23.2 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.2 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 6-10 |  |  | -23.2 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.2 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.2 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.2 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

#### 6.6.2.1A	Spectrum emission mask for CA

##### 6.6.2.1A.0	Minimum conformance requirements

For inter-band carrier aggregation with one component carrier per operating band and the uplink active in two E-UTRA bands, the spectrum emission mask of the UE is defined per component carrier while both component carriers are active and the requirements are specified in subclauses 6.6.2.1.3 and 6.6.2.2.3. If for some frequency spectrum emission masks of component carriers overlap then spectrum emission mask allowing higher power spectral density applies for that frequency. If for some frequency a component carrier spectrum emission mask overlaps with the channel bandwidth of another component carrier, then the emission mask does not apply for that frequency.

For intra-band contiguous carrier aggregation the spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the  edge of the aggregated channel bandwidth (Table 5.4.2A-1) For intra-band contiguous carrier aggregation the bandwidth class B and C, the power of any UE emission shall not exceed the levels specified in Table 6.6.2.1A.0-1 and Table 6.6.2.1A.0-2 for the specified channel bandwidth.

Table 6.6.2.1A.0-1: General E-UTRA CA spectrum emission mask for Bandwidth Class B

| Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 25RB+25RB (9.8MHz) | 25RB+50RB (14.95 MHz) | 25RB+75RB (19.8MHz) | 50RB+50RB (19.9 MHz) | Measurement bandwidth |
| 0-1 | -18 | -20 | -21 | -21 | 30 kHz |
| 1-5 | -10 | -10 | -10 | -10 | 1 MHz |
| 5-9.8 | -13 | -13 | -13 | -13 | 1 MHz |
| 9.8-14.8 | -25 | -13 | -13 | -13 | 1 MHz |
| 14.8-14.95 |  | -13 | -13 | -13 | 1 MHz |
| 14.95-19.80 |  | -25 | -13 | -13 | 1 MHz |
| 19.80-19.90 |  | -25 | -25 | -13 | 1 MHz |
| 19.90-19.95 |  | -25 | -25 | -25 | 1 MHz |
| 19.95-24.80 |  |  | -25 | -25 | 1 MHz |
| 24.80-24.90 |  |  |  | -25 | 1 MHz |

Table 6.6.2.1A.0-2: General E-UTRA CA spectrum emission mask for Bandwidth Class C

| Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 25RB+100RB (24.95MHz) | 50RB+75RB (24.75 MHz) | 50RB+100RB (29.9 MHz) | 75RB+75RB (30 MHz) | 75RB+100RB (34.85 MHz) | 100RB+100RB (39.8 MHz) | Measurement bandwidth |
| 0-1 | -22 | -22 | -22.5 | -22.5 | -23.5 | -24 | 30 kHz |
| 1-5 | -10 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 5-24.75 | -13 | -13 | -13 | -13 | -13 | -13 | 1 MHz |
| 24.75-24.95 | -13 | -25 | -13 | -13 | -13 | -13 | 1 MHz |
| 24.95-29.75 | -25 | -25 | -13 | -13 | -13 | -13 | 1 MHz |
| 29.75-29.9 | -25 |  | -13 | -13 | -13 | -13 | 1 MHz |
| 29.9-29.95 | -25 |  | -25 | -13 | -13 | -13 | 1 MHz |
| 29.95-30 |  |  | -25 | -13 | -13 | -13 | 1 MHz |
| 30-34.85 |  |  | -25 | -25 | -13 | -13 | 1 MHz |
| 34.85-34.9 |  |  | -25 | -25 | -25 | -13 | 1 MHz |
| 34.9-35 |  |  |  | -25 | -25 | -13 | 1 MHz |
| 35-39.8 |  |  |  |  | -25 | -13 | 1 MHz |
| 39.8-39.85 |  |  |  |  | -25 | -25 | 1 MHz |
| 39.85-44.8 |  |  |  |  |  | -25 | 1 MHz |

For intra-band non-contiguous carrier aggregation transmission the spectrum emission mask requirement is defined as a composite spectrum emissions mask. Composite spectrum emission mask applies to frequencies up to  ΔfOOB starting from the edges of the sub-blocks. Composite spectrum emission mask is defined as follows:

a)	Composite spectrum emission mask is a combination of individual sub-block spectrum emissions masks

b)	In case the sub-block consist of one component carrier the sub-lock general spectrum emission mask is defined in subclause 6.6.2.1.3

c)	If for some frequency sub-block spectrum emission masks overlap then spectrum emission mask allowing higher power spectral density applies for that frequency

d)	If for some frequency a sub-block spectrum emission mask overlaps with the sub-block bandwidth of another sub-block, then the emission mask does not apply for that frequency.

For combinations of intra-band and inter-band carrier aggregation with three uplink component carriers (up to two contiguously aggregated carriers per band), the spectrum emission mask of the UE is defined per E-UTRA band while all component carriers are active. For the E-UTRA band supporting one component carrier the requirements in subclauses 6.6.2.1.3 and 6.6.2.2.3 apply. For the E-UTRA band supporting two contiguous component carriers the requirements specified in subclause 6.6.2.1A.0 apply. If for some frequency spectrum emission masks of single component carrier and two contiguous component carriers overlap then spectrum emission mask allowing higher power spectral density applies for that frequency. If for some frequency spectrum emission masks of single component carrier or two contiguous component carriers overlap then the emission mask does not apply for that frequency.

The normative reference for this requirement is TS 36.101 [2] subclause 6.6.2.1A.

##### 6.6.2.1A.1	Spectrum emission mask for CA (intra-band contiguous DL CA and UL CA)

6.6.2.1A.1.1	Test purpose

To verify that the power of any UE emission shall not exceed specified lever for the specified channel bandwidth for CA.

6.6.2.1A.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 10 and forward that support intra-band contiguous DL CA and UL CA.

6.6.2.1A.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.2.1A.0.

6.6.2.1A.1.4	Test description

6.6.2.1A.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.1A.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.1A.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Low and High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| ID | PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 25 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 2 | 25 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 3 | 25 | 25 |  | QPSK | 50 | P_25@0 | S_25@0 | - | - |
| 4 | 25 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 5 | 25 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 6 | 25 | 25 |  | 16QAM | 50 | P_25@0 | S_25@0 | - | - |
| 7 | 25 | 25 |  | QPSK | 2 | P_1@0 | S_1@24 | - | - |
| 8 | 50 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 9 | 50 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 10 | 50 | 25 |  | QPSK | 75 | P_50@0 | S_25@0 | - | - |
| 11 | 50 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 12 | 50 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 13 | 50 | 25 |  | 16QAM | 75 | P_50@0 | S_25@0 | - | - |
| 14 | 50 | 25 |  | QPSK | 2 | P_1@0 | S_1@24 | - | - |
| 15 | 50 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 16 | 50 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 17 | 50 | 50 |  | QPSK | 100 | P_50@0 | S_50@0 | - | - |
| 18 | 50 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 19 | 50 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 20 | 50 | 50 |  | 16QAM | 100 | P_50@0 | S_50@0 | - | - |
| 21 | 50 | 50 |  | QPSK | 2 | P_1@0 | S_1@49 | - | - |
| 22 | 75 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 23 | 75 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 24 | 75 | 25 |  | QPSK | 100 | P_75@0 | S_25@0 | - | - |
| 25 | 75 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 26 | 75 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 27 | 75 | 25 |  | 16QAM | 100 | P_75@0 | S_25@0 | - | - |
| 28 | 75 | 25 |  | QPSK | 2 | P_1@0 | S_1@24 | - | - |
| 29 | 75 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 30 | 75 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 31 | 75 | 50 |  | QPSK | 125 | P_75@0 | S_50@0 | - | - |
| 32 | 75 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 33 | 75 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 34 | 75 | 50 |  | 16QAM | 125 | P_75@0 | S_50@0 | - | - |
| 35 | 75 | 50 |  | QPSK | 2 | P_1@0 | S_1@49 | - | - |
| 36 | 75 | 75 | N/A for this test | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 37 | 75 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 | - | - |
| 38 | 75 | 75 |  | QPSK | 150 | P_75@0 | S_75@0 | - | - |
| 39 | 75 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 | - | - |
| 40 | 75 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 | - | - |
| 41 | 75 | 75 |  | 16QAM | 150 | P_75@0 | S_75@0 | - | - |
| 42 | 75 | 75 |  | QPSK | 2 | P_1@0 | S_1@74 | - | - |
| 43 | 100 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 44 | 100 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 45 | 100 | 25 |  | QPSK | 125 | P_100@0 | S_25@0 | - | - |
| 46 | 100 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 47 | 100 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 48 | 100 | 25 |  | 16QAM | 125 | P_100@0 | S_25@0 | - | - |
| 49 | 100 | 25 |  | QPSK | 2 | P_1@0 | S_1@24 | - | - |
| 50 | 100 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 51 | 100 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 52 | 100 | 50 |  | QPSK | 150 | P_100@0 | S_50@0 | - | - |
| 53 | 100 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 54 | 100 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 55 | 100 | 50 |  | 16QAM | 150 | P_100@0 | S_50@0 | - | - |
| 56 | 100 | 50 |  | QPSK | 2 | P_1@0 | S_1@49 | - | - |
| 57 | 100 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 58 | 100 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 | - | - |
| 59 | 100 | 75 |  | QPSK | 175 | P_100@0 | S_75@0 | - | - |
| 60 | 100 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 | - | - |
| 61 | 100 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 | - | - |
| 62 | 100 | 75 |  | 16QAM | 175 | P_100@0 | S_75@0 | - | - |
| 63 | 100 | 75 |  | QPSK | 2 | P_1@0 | S_1@74 | - | - |
| 64 | 100 | 100 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| 65 | 100 | 100 |  | QPSK | 100 | P_100@0 | S_0@0 | - | - |
| 66 | 100 | 100 |  | QPSK | 200 | P_100@0 | S_100@0 | - | - |
| 67 | 100 | 100 |  | 16QAM | 18 | P_18@0 | S_0@0 | - | - |
| 68 | 100 | 100 |  | 16QAM | 100 | P_100@0 | S_0@0 | - | - |
| 69 | 100 | 100 |  | 16QAM | 200 | P_100@0 | S_100@0 | - | - |
| 70 | 100 | 100 |  | QPSK | 2 | P_1@0 | S_1@99 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] clause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.1A.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.1A.1.4.3.

6.6.2.1A.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.6.2.1A.1.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.6.2.1.4.1-1 on both PCC and SCC. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

6.	Measure the mean power over all component carriers in the CA configuration of the radio access mode, which shall meet the requirements described in Table 6.2.3A.1.5-1. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

7.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to table 6.6.2.1A.1.5-1 or 6.6.2.1A.1.5-2 and using a rms detector, as applicable. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.6.2.1A.1.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

6.6.2.1A.1.5	Test Requirements

The measured UE mean power with the aggregated channel bandwidth as specified in clause 5.4.2A, derived in step 6, shall fulfil requirements in Table 6.2.3A.1.5-1 as appropriate,

and

the power of any UE emission shall fulfil requirements in Table.6.6.2.1A.1.5-1 or Table.6.6.2.1A.1.5-2 for Bandwidth Class C and Table.6.6.2.1A.1.5-3 or Table.6.6.2.1A.1.5-4 for Bandwidth Class B, as applicable.

Table 6.6.2.1A.1.5-1: General E-UTRA spectrum emission mask for CA Bandwidth Class C, E-UTRA bands ≤ 3GHz

| Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 25RB+100RB (24.95MHz) | 50RB+75RB (24.75 MHz) | 50RB+100RB (29.9 MHz) | 75RB+75RB (30 MHz) | 75RB+100RB (34.85 MHz) | 100RB+100RB (39.8 MHz) | Measurement bandwidth |
| 0-1 | -20.5 | -20.5 | -21 | -21 | -22 | -22.5 | 30 kHz |
| 1-5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | 1 MHz |
| 5-24.75 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 24.75-24.95 |  | -23.5 |  |  |  |  | 1 MHz |
| 24.95-29.75 | -23.5 |  |  |  |  |  | 1 MHz |
| 29.75-29.9 |  |  |  |  |  |  | 1 MHz |
| 29.9-29.95 |  |  | -23.5 |  |  |  | 1 MHz |
| 29.95-30 |  |  |  |  |  |  | 1 MHz |
| 30-34.85 |  |  |  | -23.5 |  |  | 1 MHz |
| 34.85-34.9 |  |  |  |  | -23.5 |  | 1 MHz |
| 34.9-35 |  |  |  |  |  |  | 1 MHz |
| 35-39.8 |  |  |  |  |  |  | 1 MHz |
| 39.8-39.85 |  |  |  |  |  | -23.5 | 1 MHz |
| 39.85-44.8 |  |  |  |  |  |  | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the aggregated channel bandwidth and below the lower edge of the aggregated channel bandwidth. |  |  |  |  |  |  |  |

Table 6.6.2.1A.1.5-2: General E-UTRA spectrum emission mask for CA Bandwidth Class C, 3GHz < E-UTRA bands ≤ 4.2GHz

| Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 25RB+100RB (24.95MHz) | 50RB+75RB (24.75 MHz) | 50RB+100RB (29.9 MHz) | 75RB+75RB (30 MHz) | 75RB+100RB (34.85 MHz) | 100RB+100RB (39.8 MHz) | Measurement bandwidth |
| 0-1 | -20.2 | -20.2 | -20.7 | -20.7 | -21.7 | -22.2 | 30 kHz |
| 1-5 | -8.2 | -8.2 | -8.2 | -8.2 | -8.2 | -8.2 | 1 MHz |
| 5-24.75 | -11.2 | -11.2 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 24.75-24.95 |  | -23.2 |  |  |  |  | 1 MHz |
| 24.95-29.75 | -23.2 |  |  |  |  |  | 1 MHz |
| 29.75-29.9 |  |  |  |  |  |  | 1 MHz |
| 29.9-29.95 |  |  | -23.2 |  |  |  | 1 MHz |
| 29.95-30 |  |  |  |  |  |  | 1 MHz |
| 30-34.85 |  |  |  | -23.2 |  |  | 1 MHz |
| 34.85-34.9 |  |  |  |  | -23.2 |  | 1 MHz |
| 34.9-35 |  |  |  |  |  |  | 1 MHz |
| 35-39.8 |  |  |  |  |  |  | 1 MHz |
| 39.8-39.85 |  |  |  |  |  | -23.2 | 1 MHz |
| 39.85-44.8 |  |  |  |  |  |  | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the aggregated channel bandwidth and below the lower edge of the aggregated channel bandwidth. |  |  |  |  |  |  |  |

Table 6.6.2.1A.1.5-3: General E-UTRA spectrum emission mask for CA Bandwidth Class B, E-UTRA bands ≤ 3GHz

| Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 25RB+25RB (9.8MHz) | 25RB+50RB (14.95 MHz) | 25RB+75RB (19.8MHz) | 50RB+50RB (19.9 MHz) | Measurement bandwidth |
| 0-1 | -16.5 | -18.5 | -19.5 | -19.5 | 30 kHz |
| 1-5 | -8.5 | -8.5 | -8.5 | -8.5 | 1 MHz |
| 5-9.8 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 9.8-14.8 | -23.5 |  |  |  | 1 MHz |
| 14.8-14.95 |  |  |  |  | 1 MHz |
| 14.95-19.80 |  | -23.5 |  |  | 1 MHz |
| 19.80-19.90 |  |  | -23.5 |  | 1 MHz |
| 19.90-19.95 |  |  |  | -23.5 | 1 MHz |
| 19.95-24.80 |  |  |  |  | 1 MHz |
| 24.80-24.90 |  |  |  |  | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the aggregated channel bandwidth and below the lower edge of the aggregated channel bandwidth. |  |  |  |  |  |

Table 6.6.2.1A.1.5-4: General E-UTRA spectrum emission mask for CA Bandwidth Class B, 3GHz < E-UTRA bands ≤ 4.2GHz

| Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 25RB+25RB (9.8MHz) | 25RB+50RB (14.95 MHz) | 25RB+75RB (19.8MHz) | 50RB+50RB (19.9 MHz) | Measurement bandwidth |
| 0-1 | -16.2 | -18.2 | -19.2 | -19.2 | 30 kHz |
| 1-5 | -8.2 | -8.2 | -8.2 | -8.2 | 1 MHz |
| 5-9.8 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 9.8-14.8 | -23.2 |  |  |  | 1 MHz |
| 14.8-14.95 |  |  |  |  | 1 MHz |
| 14.95-19.80 |  | -23.2 |  |  | 1 MHz |
| 19.80-19.90 |  |  | -23.2 |  | 1 MHz |
| 19.90-19.95 |  |  |  | -23.2 | 1 MHz |
| 19.95-24.80 |  |  |  |  | 1 MHz |
| 24.80-24.90 |  |  |  |  | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the aggregated channel bandwidth and below the lower edge of the aggregated channel bandwidth. |  |  |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

##### 6.6.2.1A.2	Spectrum emission mask for CA (inter-band DL CA and UL CA)

6.6.2.1A.2.1	Test purpose

To verify that the power of any UE emission shall not exceed specified lever for the specified channel bandwidth for CA.

6.6.2.1A.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 11 and forward that support inter-band DL CA and UL CA.

6.6.2.1A.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.2.1A.0.

6.6.2.1A.2.4	Test description

6.6.2.1A.2.4.1	Initial condition

Same as in clause 6.6.2.1A.1.4.1 with the following exceptions:

-	Instead of Table 5.4.2A.1-1 use Table 5.4.2A.1-2 for CA configurations with two bands or Table 5.4.2A.1-2a for CA configurations with three bands.

-	Instead of Table 6.6.2.1A.1.4.1-1  use Table 6.6.2.1A.2.4.1-1.

-	Instead of clause 6.6.2.1A.1.4.3 use clause 6.6.2.1A.2.4.3.

Table 6.6.2.1A.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 6 | 25 | N/Afor this test | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 6 | 25 |  | QPSK | 2 | P_1@5 | S_1@24 | - | - |
| 6 | 25 |  | QPSK | 13 | P_5@0 | S_8@0 | - | - |
| 6 | 25 |  | QPSK | 13 | P_5@1 | S_8@17 | - | - |
| 6 | 25 |  | 16QAM | 2 | P_1@0 | S_1@0 |  |  |
| 6 | 25 |  | 16QAM | 2 | P_1@5 | S_1@24 |  |  |
| 6 | 25 |  | 16QAM | 13 | P_5@0 | S_8@0 |  |  |
| 6 | 25 |  | 16QAM | 13 | P_5@1 | S_8@17 |  |  |
| 6 | 50 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 6 | 50 |  | QPSK | 2 | P_1@5 | S_1@49 | - | - |
| 6 | 50 |  | QPSK | 17 | P_5@0 | S_12@0 | - | - |
| 6 | 50 |  | QPSK | 17 | P_5@1 | S_12@38 | - | - |
| 6 | 50 |  | 16QAM | 2 | P_1@0 | S_1@0 | - | - |
| 6 | 50 |  | 16QAM | 2 | P_1@5 | S_1@49 | - | - |
| 6 | 50 |  | 16QAM | 17 | P_5@0 | S_12@0 | - | - |
| 6 | 50 |  | 16QAM | 17 | P_5@1 | S_12@38 | - | - |
| 15 | 25 |  | QPSK | 2 | S_1@0 | S_1@0 |  |  |
| 15 | 25 |  | QPSK | 2 | S_1@14 | S_1@24 |  |  |
| 15 | 25 |  | QPSK | 12 | S_4@0 | S_8@0 |  |  |
| 15 | 25 |  | QPSK | 12 | S_4@11 | S_8@17 |  |  |
| 15 | 25 |  | 16QAM | 2 | S_1@0 | S_1@0 |  |  |
| 15 | 25 |  | 16QAM | 2 | S_1@14 | S_1@24 |  |  |
| 15 | 25 |  | 16QAM | 12 | S_4@0 | S_8@0 |  |  |
| 15 | 25 |  | 16QAM | 12 | S_4@11 | S_8@17 |  |  |
| 25 | 6 |  | QPSK | 2 | S_1@0 | P_1@0 |  |  |
| 25 | 6 |  | QPSK | 2 | S_1@24 | P_1@5 |  |  |
| 25 | 6 |  | QPSK | 13 | S_8@0 | P_5@0 |  |  |
| 25 | 6 |  | QPSK | 13 | S_8@17 | P_5@1 |  |  |
| 25 | 6 |  | 16QAM | 2 | S_1@0 | P_1@0 |  |  |
| 25 | 6 |  | 16QAM | 2 | S_1@24 | P_1@5 |  |  |
| 25 | 6 |  | 16QAM | 13 | S_8@0 | P_5@0 |  |  |
| 25 | 6 |  | 16QAM | 13 | S_8@17 | P_5@1 |  |  |
| 25 | 15 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 25 | 15 |  | QPSK | 2 | P_1@24 | S_1@14 | - | - |
| 25 | 15 |  | QPSK | 12 | P_8@0 | S_4@0 | - | - |
| 25 | 15 |  | QPSK | 12 | P_8@17 | S_4@11 | - | - |
| 25 | 15 |  | 16QAM | 2 | P_1@0 | S_1@0 | - | - |
| 25 | 15 |  | 16QAM | 2 | P_1@24 | S_1@14 | - | - |
| 25 | 15 |  | 16QAM | 12 | P_8@0 | S_4@0 | - | - |
| 25 | 15 |  | 16QAM | 12 | P_8@17 | S_4@11 | - | - |
| 25 | 25 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 25 | 25 |  | QPSK | 2 | P_1@24 | S_1@24 | - | - |
| 25 | 25 |  | QPSK | 16 | P_8@0 | S_8@0 | - | - |
| 25 | 25 |  | QPSK | 16 | P_8@17 | S_8@17 | - | - |
| 25 | 25 |  | 16QAM | 2 | P_1@0 | S_1@0 | - | - |
| 25 | 25 |  | 16QAM | 2 | P_1@24 | S_1@24 | - | - |
| 25 | 25 |  | 16QAM | 16 | P_8@0 | S_8@0 | - | - |
| 25 | 25 |  | 16QAM | 16 | P_8@17 | S_8@17 | - | - |
| 25 | 50 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 25 | 50 |  | QPSK | 2 | P_1@24 | S_1@49 | - | - |
| 25 | 50 |  | QPSK | 20 | P_8@0 | S_12@0 | - | - |
| 25 | 50 |  | QPSK | 20 | P_8@17 | S_12@38 | - | - |
| 25 | 50 |  | 16QAM | 2 | P_1@0 | S_1@0 | - | - |
| 25 | 50 |  | 16QAM | 2 | P_1@24 | S_1@49 | - | - |
| 25 | 50 |  | 16QAM | 20 | P_8@0 | S_12@0 | - | - |
| 25 | 50 |  | 16QAM | 20 | P_8@17 | S_12@38 | - | - |
| 50 | 50 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 50 | 50 |  | QPSK | 2 | P_1@49 | S_1@49 | - | - |
| 50 | 50 |  | QPSK | 24 | P_12@0 | S_12@0 | - | - |
| 50 | 50 |  | QPSK | 24 | P_12@38 | S_12@38 | - | - |
| 50 | 50 |  | 16QAM | 2 | P_1@0 | S_1@0 | - | - |
| 50 | 50 |  | 16QAM | 2 | P_1@49 | S_1@49 | - | - |
| 50 | 50 |  | 16QAM | 24 | P_12@0 | S_12@0 | - | - |
| 50 | 50 |  | 16QAM | 24 | P_12@38 | S_12@38 | - | - |
| 50 | 100 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 50 | 100 |  | QPSK | 2 | P_1@49 | S_1@99 | - | - |
| 50 | 100 |  | QPSK | 30 | P_12@0 | S_18@0 | - | - |
| 50 | 100 |  | QPSK | 30 | P_12@38 | S_18@82 | - | - |
| 50 | 100 |  | 16QAM | 2 | P_1@0 | S_1@0 | - | - |
| 50 | 100 |  | 16QAM | 2 | P_1@49 | S_1@99 | - | - |
| 50 | 100 |  | 16QAM | 30 | P_12@0 | S_18@0 | - | - |
| 50 | 100 |  | 16QAM | 30 | P_12@38 | S_18@82 | - | - |
| 75 | 25 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 75 | 25 |  | QPSK | 2 | P_1@74 | S_1@24 | - | - |
| 75 | 25 |  | QPSK | 24 | P_16@0 | S_8@0 | - | - |
| 75 | 25 |  | QPSK | 24 | P_16@59 | S_8@17 | - | - |
| 75 | 25 |  | 16QAM | 2 | P_1@0 | S_1@0 | - | - |
| 75 | 25 |  | 16QAM | 2 | P_1@74 | S_1@24 | - | - |
| 75 | 25 |  | 16QAM | 24 | P_16@0 | S_8@0 | - | - |
| 75 | 25 |  | 16QAM | 24 | P_16@59 | S_8@17 | - | - |
| 75 | 75 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 75 | 75 |  | QPSK | 2 | P_1@74 | S_1@74 | - | - |
| 75 | 75 |  | QPSK | 32 | P_16@0 | S_16@0 | - | - |
| 75 | 75 |  | QPSK | 32 | P_16@59 | S_16@59 | - | - |
| 75 | 75 |  | 16QAM | 2 | P_1@0 | S_1@0 | - | - |
| 75 | 75 |  | 16QAM | 2 | P_1@74 | S_1@74 | - | - |
| 75 | 75 |  | 16QAM | 32 | P_16@0 | S_16@0 | - | - |
| 75 | 75 |  | 16QAM | 32 | P_16@59 | S_16@59 | - | - |
| 100 | 50 |  | QPSK | 2 | P_1@0 | S_1@0 |  |  |
| 100 | 50 |  | QPSK | 2 | P_1@99 | S_1@49 |  |  |
| 100 | 50 |  | QPSK | 30 | P_18@0 | S_12@0 |  |  |
| 100 | 50 |  | QPSK | 30 | P_18@82 | S_12@38 |  |  |
| 100 | 50 |  | 16QAM | 2 | P_1@0 | S_1@0 |  |  |
| 100 | 50 |  | 16QAM | 2 | P_1@99 | S_1@49 |  |  |
| 100 | 50 |  | 16QAM | 30 | P_18@0 | S_12@0 |  |  |
| 100 | 50 |  | 16QAM | 30 | P_18@82 | S_12@38 |  |  |
| 100 | 75 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 100 | 75 |  | QPSK | 2 | P_1@99 | S_1@74 | - | - |
| 100 | 75 |  | QPSK | 34 | P_18@0 | S_16@0 | - | - |
| 100 | 75 |  | QPSK | 34 | P_18@82 | S_16@59 | - | - |
| 100 | 75 |  | 16QAM | 2 | P_1@0 | S_1@0 | - | - |
| 100 | 75 |  | 16QAM | 2 | P_1@99 | S_1@74 | - | - |
| 100 | 75 |  | 16QAM | 34 | P_18@0 | S_16@0 | - | - |
| 100 | 75 |  | 16QAM | 34 | P_18@82 | S_16@59 | - | - |
| 100 | 100 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 100 | 100 |  | QPSK | 2 | P_1@99 | S_1@99 | - | - |
| 100 | 100 |  | QPSK | 36 | P_18@0 | S_18@0 | - | - |
| 100 | 100 |  | QPSK | 36 | P_18@82 | S_18@82 | - | - |
| 100 | 100 |  | 16QAM | 2 | P_1@0 | S_1@0 | - | - |
| 100 | 100 |  | 16QAM | 2 | P_1@99 | S_1@99 | - | - |
| 100 | 100 |  | 16QAM | 36 | P_18@0 | S_18@0 | - | - |
| 100 | 100 |  | 16QAM | 36 | P_18@82 | S_18@82 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |

6.6.2.1A.2.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.6.2.1A.1.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.6.2.1.4.1-1 on both PCC and SCC. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

6.	Measure the mean power for each CC, which shall meet the requirements described in Table 6.2.3A.2.5-1. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

7.	Measure the power of the transmitted signal for PCC with a measurement filter of bandwidths according to table 6.6.2.1A.2.5-1 or 6.6.2.1A.2.5-2 and using a rms detector, as applicable. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

8.	Measure the power of the transmitted signal for SCC with a measurement filter of bandwidths according to table 6.6.2.1A.2.5-1 or 6.6.2.1A.2.5-2 and using a rms detector, as applicable. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.6.2.1A.2.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

6.6.2.1A.2.5	Test Requirements

Editor's note:	Need further investigation for test requirements in case some frequency spectrum emission masks of component carriers overlap for future coming inter-band UL CA combinations

The measured UE mean power in the channel bandwidth for each CC, derived in step 6, shall fulfil requirements in Table 6.2.3A.2.5-1 as appropriate, and the power of any UE emission per component carrier derived in step 7 and 8 shall fulfil requirements in Table.6.6.2.1A.2.5-1 or Table.6.6.2.1A.2.5-2, as applicable.

Table 6.6.2.1A.2.5-1: General E-UTRA spectrum emission mask, E-UTRA bands ≤ 3GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.5 | -11.5 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 1-2.5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 6-10 |  |  | -23.5 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.5 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.5 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

Table 6.6.2.1A.2.5-2: General E-UTRA spectrum emission mask, 3GHz < E-UTRA bands ≤ 4.2GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.2 | -11.2 | -13.2 | -16.2 | -18.2 | -19.2 | 30 kHz |
| 1-2.5 | -8.2 | -8.2 | -8.2 | -8.2 | -8.2 | -8.2 | 1 MHz |
| 2.5-2.8 | -23.2 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.2 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 6-10 |  |  | -23.2 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.2 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.2 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.2 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

##### 6.6.2.1A.3	Spectrum emission mask for CA (intra-band non-contiguous DL CA and UL CA)

6.6.2.1A.3.1	Test purpose

To verify that the power of any UE emission shall not exceed specified lever for the specified channel bandwidth for CA.

6.6.2.1A.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 11 and forward that support intra-band non-contiguous DL CA and UL CA.

6.6.2.1A.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.2.1A.0.

6.6.2.1A.3.4	Test description

6.6.2.1A.3.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-3. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.1A.3.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.1A.3.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Refer to test points A: Low Wgap, Maximum Wgap |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  | Refer to test point Test only test points with Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  | Wgap [MHz] | DL Allocation |  | UL Allocation |  |  |  |
|  | PCC NRB | SCCs NRB |  | CC MOD | PCC & SCC RB allocation | CC MOD | PCC NRB_alloc (LCRB @ RBstart) | CC MOD | SCC NRB_alloc (LCRB @ RBstart) |
| Test Parameters for CA_4A-4A Configurations |  |  |  |  |  |  |  |  |  |
| 1 | 25 | 25 | 35 | N/A | N/A | 16QAM | P_1@0 | 16QAM | S_1@24 |
| 2 | 25 | 25 | 35 | N/A | N/A | 16QAM | P_25@0 | 16QAM | S_25@0 |
| 3 | 25 | 25 | 5 | N/A | N/A | 16QAM | P_1@0 | 16QAM | S_1@24 |
| 4 | 25 | 25 | 5 | N/A | N/A | 16QAM | P_25@0 | 16QAM | S_25@0 |
| 5 | 100 | 100 | 5 | N/A | N/A | 16QAM | P_1@0 | 16QAM | S_1@99 |
| 6 | 100 | 100 | 5 | N/A | N/A | 16QAM | P_25@0 | 16QAM | S_25@75 |
| 7 | 100 | 100 | 5 | N/A | N/A | 16QAM | P_100@0 | 16QAM | S_100@0 |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] clause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.1A.3.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.1A.3.4.3.

6.6.2.1A.3.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels except PHICH.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.6.2.1A.3.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.6.2.1.4.1-1 on both PCC and SCC. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

6.	Measure the mean power over all component carriers in the CA configuration of the radio access mode, which shall meet the requirements described in Table 6.2.3A.1.5-1. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

7.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to table 6.6.2.1A.3.5-1 or 6.6.2.1A.3.5-2 and using a rms detector, as applicable. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.6.2.1A.3.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

6.6.2.1A.3.5	Test Requirements

The measured UE mean power with the aggregated channel bandwidth as specified in clause 5.4.2A, derived in step 6, shall fulfil requirements in Table 6.2.3A.1.5-1 as appropriate,

and

the power of any UE emission shall fulfil requirements in Table.6.6.2.1A.3.5-1.

Table 6.6.2.1A.3.5-1: E-UTRA spectrum emission mask for CA_4A-4A

| Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25RB+25RB (Max Gap) |  |  | 25RB+25RB (Mid range) |  |  | 100RB+100RB |  |  |
| Frequency ranges[MHz] | Limit [dBm] | MBW [MHz] | Frequency ranges[MHz] | Limit [dBm] | MBW | Frequency ranges[MHz] | Limit [dBm] | MBW [MHz] |
| 1700-1704 | -23.5 | 1 | 1715-1719 | -23.5 | 1 | 1685-1690 | -23.5 | 1 |
| 1704-1705 | -11.5 | 1 | 1719-1720 | -11.5 | 1 | 1690-1705 | -11.5 | 1 |
| 1705-1709 | -8.5 | 1 | 1720-1724 | -8.5 | 1 | 1705-1709 | -8.5 | 1 |
| 1709-1710 | -13.5 | 0.03 | 1724-1725 | -13.5 | 0.03 | 1709-1710 | -19.5 | 0.03 |
| - | - | - | - | - | - | - | - | - |
| 1715-1716 | -13.5 | 0.03 | 1730-1731 | -13.5 | 0.03 | 1730-1731 | -19.5 | 0.03 |
| 1716-1720 | -8.5 | 1 | 1732-1734 | -8.5 | 1 | 1731-1734 | -8.5 | 1 |
| 1720-1721 | -11.5 | 1 | 1734-1735 | -13.5 | 0.03 | 1734-1735 | -19.5 | 0.03 |
| 1721-1725 | -23.5 | 1 | - | - | - | - | - | - |
| 1740-1744 | -23.5 | 1 | 1741-1745 | -8.5 | 1 | 1756-1760 | -8.5 | 1 |
| 1744-1745 | -11.5 | 1 | 1745-1746 | -11.5 | 1 | 1760-1775 | -11.5 | 1 |
| 1745-1749 | -8.5 | 1 | 1746-1750 | -23.5 | 1 | 1775-1780 | -23.5 | 1 |
| 1749-1750 | -13.5 | 0.03 |  |  |  |  |  |  |
| - | - | - |  |  |  |  |  |  |
| 1755-1756 | -13.5 | 0.03 |  |  |  |  |  |  |
| 1756-1760 | -8.5 | 1 |  |  |  |  |  |  |
| 1760-1761 | -11.5 | 1 |  |  |  |  |  |  |
| 1761-1765 | -23.5 | 1 |  |  |  |  |  |  |
| Note 1: At the boundary of spectrum emission limit, the first and last measurement position with a 30kHz filter is the inside of +0.015MHz and -0.015MHz, respectively. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: MBW denotes Measurement Bandwidth with which the requirement has to be met. |  |  |  |  |  |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

##### 6.6.2.1A.4	Spectrum emission mask for CA (3UL CA)

6.6.2.1A.4.1	Test purpose

To verify that the power of any UE emission shall not exceed specified lever for the specified channel bandwidth for CA.

6.6.2.1A.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 13 and forward that support intra-band contiguous or inter-band and 3UL CA.

6.6.2.1A.4.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.2.1A.0.

6.6.2.1A.4.4	Test description

6.6.2.1A.4.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-3. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and are shown in table 6.6.2.1A.4.4.1-1 & 6.6.2.1A.4.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.1A.4.4.1-1: Test Configuration Table for intra-band contiguous CA

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Low and High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| ID | PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 100 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 |  |
| 2 | 100 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | S_0@0 |  |
| 3 | 100 | 25 |  | QPSK | 150 | P_100@0 | S_25@0 | S_25@0 |  |
| 4 | 100 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | S_0@0 |  |
| 5 | 100 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | S_0@0 |  |
| 6 | 100 | 25 |  | 16QAM | 150 | P_100@0 | S_25@0 | S_25@0 |  |
| 7 | 100 | 25 |  | QPSK | 3 | P_1@0 | S_1@24 | S_1@24 |  |
| 8 | 75 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | S_0@0 |  |
| 9 | 75 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 | S_0@0 |  |
| 10 | 75 | 75 |  | QPSK | 225 | P_75@0 | S_75@0 | S_75@0 |  |
| 11 | 75 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 | S_0@0 |  |
| 12 | 75 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 | S_0@0 |  |
| 13 | 75 | 75 |  | 16QAM | 225 | P_75@0 | S_75@0 | S_75@0 |  |
| 14 | 75 | 75 |  | QPSK | 3 | P_1@0 | S_1@74 | S_1@74 |  |
| 15 | 100 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 |  |
| 16 | 100 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | S_0@0 |  |
| 17 | 100 | 50 |  | QPSK | 200 | P_100@0 | S_50@0 | S_50@0 |  |
| 18 | 100 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | S_0@0 |  |
| 19 | 100 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 | S_0@0 |  |
| 20 | 100 | 50 |  | 16QAM | 200 | P_100@0 | S_50@0 | S_50@0 |  |
| 21 | 100 | 50 |  | QPSK | 3 | P_1@0 | S_1@49 | S_1@49 |  |
| 22 | 100 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | S_0@0 |  |
| 23 | 100 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 | S_0@0 |  |
| 24 | 100 | 75 |  | QPSK | 250 | P_100@0 | S_75@0 | S_75@0 |  |
| 25 | 100 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 | S_0@0 |  |
| 26 | 100 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 | S_0@0 |  |
| 27 | 100 | 75 |  | 16QAM | 250 | P_100@0 | S_75@0 | S_75@0 |  |
| 28 | 100 | 75 |  | QPSK | 3 | P_1@0 | S_1@74 | S_1@74 |  |
| 29 | 100 | 100 |  | QPSK | 18 | P_18@0 | S_0@0 | S_0@0 |  |
| 30 | 100 | 100 |  | QPSK | 100 | P_100@0 | S_0@0 | S_0@0 |  |
| 31 | 100 | 100 |  | QPSK | 300 | P_100@0 | S_100@0 | S_100@0 |  |
| 32 | 100 | 100 |  | 16QAM | 18 | P_18@0 | S_0@0 | S_0@0 |  |
| 33 | 100 | 100 |  | 16QAM | 100 | P_100@0 | S_0@0 | S_0@0 |  |
| 34 | 100 | 100 |  | 16QAM | 300 | P_100@0 | S_100@0 | S_100@0 |  |
| 35 | 100 | 100 |  | QPSK | 3 | P_1@0 | S_1@99 | S_1@99 |  |
| 36 | 25 | 50 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 |  |
| 37 | 25 | 50 |  | QPSK | 25 | P_25@0 | S_0@0 | S_0@0 |  |
| 38 | 25 | 50 |  | QPSK | 125 | P_25@0 | S_50@0 | S_50@0 |  |
| 39 | 25 | 50 |  | 16QAM | 8 | P_8@0 | S_0@0 | S_0@0 |  |
| 40 | 25 | 50 |  | 16QAM | 25 | P_25@0 | S_0@0 | S_0@0 |  |
| 41 | 25 | 50 |  | 16QAM | 125 | P_25@0 | S_50@0 | S_50@0 |  |
| 42 | 25 | 50 |  | QPSK | 3 | P_1@0 | S_1@49 | S_1@49 |  |
| 43 | 50 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | S_0@0 |  |
| 44 | 50 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | S_0@0 |  |
| 45 | 50 | 50 |  | QPSK | 150 | P_50@0 | S_50@0 | S_50@0 |  |
| 46 | 50 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | S_0@0 |  |
| 47 | 50 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 | S_0@0 |  |
| 48 | 50 | 50 |  | 16QAM | 150 | P_50@0 | S_50@0 | S_50@0 |  |
| 49 | 50 | 50 |  | QPSK | 3 | P_1@0 | S_1@49 | S_1@49 |  |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |  |

Table 6.6.2.1A.4.4.1-2: Test Configuration Table for inter-band CA

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 6 | 25 | N/Afor this test | QPSK | 3 | P_1@0 | S_1@0 | S_1@0 | - |
| 6 | 25 |  | QPSK | 3 | P_1@5 | S_1@24 | S_1@24 | - |
| 6 | 25 |  | QPSK | 21 | P_5@0 | S_8@0 | S_8@0 | - |
| 6 | 25 |  | QPSK | 21 | P_5@1 | S_8@17 | S_8@17 | - |
| 6 | 25 |  | 16QAM | 3 | P_1@0 | S_1@0 | S_1@0 |  |
| 6 | 25 |  | 16QAM | 3 | P_1@5 | S_1@24 | S_1@24 |  |
| 6 | 25 |  | 16QAM | 21 | P_5@0 | S_8@0 | S_8@0 |  |
| 6 | 25 |  | 16QAM | 21 | P_5@1 | S_8@17 | S_8@17 |  |
| 6 | 50 |  | QPSK | 3 | P_1@0 | S_1@0 | S_1@0 | - |
| 6 | 50 |  | QPSK | 3 | P_1@5 | S_1@49 | S_1@49 | - |
| 6 | 50 |  | QPSK | 29 | P_5@0 | S_12@0 | S_12@0 | - |
| 6 | 50 |  | QPSK | 29 | P_5@1 | S_12@38 | S_12@38 | - |
| 6 | 50 |  | 16QAM | 3 | P_1@0 | S_1@0 | S_1@0 | - |
| 6 | 50 |  | 16QAM | 3 | P_1@5 | S_1@49 | S_1@49 | - |
| 6 | 50 |  | 16QAM | 29 | P_5@0 | S_12@0 | S_12@0 | - |
| 6 | 50 |  | 16QAM | 29 | P_5@1 | S_12@38 | S_12@38 | - |
| 15 | 25 |  | QPSK | 3 | S_1@0 | S_1@0 | S_1@0 |  |
| 15 | 25 |  | QPSK | 3 | S_1@14 | S_1@24 | S_1@24 |  |
| 15 | 25 |  | QPSK | 20 | S_4@0 | S_8@0 | S_8@0 |  |
| 15 | 25 |  | QPSK | 20 | S_4@11 | S_8@17 | S_8@17 |  |
| 15 | 25 |  | 16QAM | 3 | S_1@0 | S_1@0 | S_1@0 |  |
| 15 | 25 |  | 16QAM | 3 | S_1@14 | S_1@24 | S_1@24 |  |
| 15 | 25 |  | 16QAM | 20 | S_4@0 | S_8@0 | S_8@0 |  |
| 15 | 25 |  | 16QAM | 20 | S_4@11 | S_8@17 | S_8@17 |  |
| 25 | 6 |  | QPSK | 3 | S_1@0 | P_1@0 | P_1@0 |  |
| 25 | 6 |  | QPSK | 3 | S_1@24 | P_1@5 | P_1@5 |  |
| 25 | 6 |  | QPSK | 18 | S_8@0 | P_5@0 | P_5@0 |  |
| 25 | 6 |  | QPSK | 18 | S_8@17 | P_5@1 | P_5@1 |  |
| 25 | 6 |  | 16QAM | 3 | S_1@0 | P_1@0 | P_1@0 |  |
| 25 | 6 |  | 16QAM | 3 | S_1@24 | P_1@5 | P_1@5 |  |
| 25 | 6 |  | 16QAM | 18 | S_8@0 | P_5@0 | P_5@0 |  |
| 25 | 6 |  | 16QAM | 18 | S_8@17 | P_5@1 | P_5@1 |  |
| 25 | 15 |  | QPSK | 3 | P_1@0 | S_1@0 | S_1@0 | - |
| 25 | 15 |  | QPSK | 3 | P_1@24 | S_1@14 | S_1@14 | - |
| 25 | 15 |  | QPSK | 16 | P_8@0 | S_4@0 | S_4@0 | - |
| 25 | 15 |  | QPSK | 16 | P_8@17 | S_4@11 | S_4@11 | - |
| 25 | 15 |  | 16QAM | 3 | P_1@0 | S_1@0 | S_1@0 | - |
| 25 | 15 |  | 16QAM | 3 | P_1@24 | S_1@14 | S_1@14 | - |
| 25 | 15 |  | 16QAM | 16 | P_8@0 | S_4@0 | S_4@0 | - |
| 25 | 15 |  | 16QAM | 16 | P_8@17 | S_4@11 | S_4@11 | - |
| 25 | 25 |  | QPSK | 3 | P_1@0 | S_1@0 | S_1@0 | - |
| 25 | 25 |  | QPSK | 3 | P_1@24 | S_1@24 | S_1@24 | - |
| 25 | 25 |  | QPSK | 24 | P_8@0 | S_8@0 | S_8@0 | - |
| 25 | 25 |  | QPSK | 24 | P_8@17 | S_8@17 | S_8@17 | - |
| 25 | 25 |  | 16QAM | 3 | P_1@0 | S_1@0 | S_1@0 | - |
| 25 | 25 |  | 16QAM | 3 | P_1@24 | S_1@24 | S_1@24 | - |
| 25 | 25 |  | 16QAM | 24 | P_8@0 | S_8@0 | S_8@0 | - |
| 25 | 25 |  | 16QAM | 24 | P_8@17 | S_8@17 | S_8@17 | - |
| 25 | 50 |  | QPSK | 3 | P_1@0 | S_1@0 | S_1@0 | - |
| 25 | 50 |  | QPSK | 3 | P_1@24 | S_1@49 | S_1@49 | - |
| 25 | 50 |  | QPSK | 32 | P_8@0 | S_12@0 | S_12@0 | - |
| 25 | 50 |  | QPSK | 32 | P_8@17 | S_12@38 | S_12@38 | - |
| 25 | 50 |  | 16QAM | 3 | P_1@0 | S_1@0 | S_1@0 | - |
| 25 | 50 |  | 16QAM | 3 | P_1@24 | S_1@49 | S_1@49 | - |
| 25 | 50 |  | 16QAM | 32 | P_8@0 | S_12@0 | S_12@0 | - |
| 25 | 50 |  | 16QAM | 32 | P_8@17 | S_12@38 | S_12@38 | - |
| 50 | 50 |  | QPSK | 3 | P_1@0 | S_1@0 | S_1@0 | - |
| 50 | 50 |  | QPSK | 3 | P_1@49 | S_1@49 | S_1@49 | - |
| 50 | 50 |  | QPSK | 36 | P_12@0 | S_12@0 | S_12@0 | - |
| 50 | 50 |  | QPSK | 36 | P_12@38 | S_12@38 | S_12@38 | - |
| 50 | 50 |  | 16QAM | 3 | P_1@0 | S_1@0 | S_1@0 | - |
| 50 | 50 |  | 16QAM | 3 | P_1@49 | S_1@49 | S_1@49 | - |
| 50 | 50 |  | 16QAM | 36 | P_12@0 | S_12@0 | S_12@0 | - |
| 50 | 50 |  | 16QAM | 36 | P_12@38 | S_12@38 | S_12@38 | - |
| 50 | 100 |  | QPSK | 3 | P_1@0 | S_1@0 | S_1@0 | - |
| 50 | 100 |  | QPSK | 3 | P_1@49 | S_1@99 | S_1@99 | - |
| 50 | 100 |  | QPSK | 48 | P_12@0 | S_18@0 | S_18@0 | - |
| 50 | 100 |  | QPSK | 48 | P_12@38 | S_18@82 | S_18@82 | - |
| 50 | 100 |  | 16QAM | 3 | P_1@0 | S_1@0 | S_1@0 | - |
| 50 | 100 |  | 16QAM | 3 | P_1@49 | S_1@99 | S_1@99 | - |
| 50 | 100 |  | 16QAM | 48 | P_12@0 | S_18@0 | S_18@0 | - |
| 50 | 100 |  | 16QAM | 48 | P_12@38 | S_18@82 | S_18@82 | - |
| 75 | 25 |  | QPSK | 3 | P_1@0 | S_1@0 | S_1@0 | - |
| 75 | 25 |  | QPSK | 3 | P_1@74 | S_1@24 | S_1@24 | - |
| 75 | 25 |  | QPSK | 32 | P_16@0 | S_8@0 | S_8@0 | - |
| 75 | 25 |  | QPSK | 32 | P_16@59 | S_8@17 | S_8@17 | - |
| 75 | 25 |  | 16QAM | 3 | P_1@0 | S_1@0 | S_1@0 | - |
| 75 | 25 |  | 16QAM | 3 | P_1@74 | S_1@24 | S_1@24 | - |
| 75 | 25 |  | 16QAM | 32 | P_16@0 | S_8@0 | S_8@0 | - |
| 75 | 25 |  | 16QAM | 32 | P_16@59 | S_8@17 | S_8@17 | - |
| 75 | 75 |  | QPSK | 3 | P_1@0 | S_1@0 | S_1@0 | - |
| 75 | 75 |  | QPSK | 3 | P_1@74 | S_1@74 | S_1@74 | - |
| 75 | 75 |  | QPSK | 48 | P_16@0 | S_16@0 | S_16@0 | - |
| 75 | 75 |  | QPSK | 48 | P_16@59 | S_16@59 | S_16@59 | - |
| 75 | 75 |  | 16QAM | 3 | P_1@0 | S_1@0 | S_1@0 | - |
| 75 | 75 |  | 16QAM | 3 | P_1@74 | S_1@74 | S_1@74 | - |
| 75 | 75 |  | 16QAM | 48 | P_16@0 | S_16@0 | S_16@0 | - |
| 75 | 75 |  | 16QAM | 48 | P_16@59 | S_16@59 | S_16@59 | - |
| 100 | 50 |  | QPSK | 3 | P_1@0 | S_1@0 | S_1@0 |  |
| 100 | 50 |  | QPSK | 3 | P_1@99 | S_1@49 | S_1@49 |  |
| 100 | 50 |  | QPSK | 42 | P_18@0 | S_12@0 | S_12@0 |  |
| 100 | 50 |  | QPSK | 42 | P_18@82 | S_12@38 | S_12@38 |  |
| 100 | 50 |  | 16QAM | 3 | P_1@0 | S_1@0 | S_1@0 |  |
| 100 | 50 |  | 16QAM | 3 | P_1@99 | S_1@49 | S_1@49 |  |
| 100 | 50 |  | 16QAM | 42 | P_18@0 | S_12@0 | S_12@0 |  |
| 100 | 50 |  | 16QAM | 42 | P_18@82 | S_12@38 | S_12@38 |  |
| 100 | 75 |  | QPSK | 3 | P_1@0 | S_1@0 | S_1@0 | - |
| 100 | 75 |  | QPSK | 3 | P_1@99 | S_1@74 | S_1@74 | - |
| 100 | 75 |  | QPSK | 50 | P_18@0 | S_16@0 | S_16@0 | - |
| 100 | 75 |  | QPSK | 50 | P_18@82 | S_16@59 | S_16@59 | - |
| 100 | 75 |  | 16QAM | 3 | P_1@0 | S_1@0 | S_1@0 | - |
| 100 | 75 |  | 16QAM | 3 | P_1@99 | S_1@74 | S_1@74 | - |
| 100 | 75 |  | 16QAM | 50 | P_18@0 | S_16@0 | S_16@0 | - |
| 100 | 75 |  | 16QAM | 50 | P_18@82 | S_16@59 | S_16@59 | - |
| 100 | 100 |  | QPSK | 3 | P_1@0 | S_1@0 | S_1@0 | - |
| 100 | 100 |  | QPSK | 3 | P_1@99 | S_1@99 | S_1@99 | - |
| 100 | 100 |  | QPSK | 54 | P_18@0 | S_18@0 | S_18@0 | - |
| 100 | 100 |  | QPSK | 54 | P_18@82 | S_18@82 | S_18@82 | - |
| 100 | 100 |  | 16QAM | 3 | P_1@0 | S_1@0 | S_1@0 | - |
| 100 | 100 |  | 16QAM | 3 | P_1@99 | S_1@99 | S_1@99 | - |
| 100 | 100 |  | 16QAM | 54 | P_18@0 | S_18@0 | S_18@0 | - |
| 100 | 100 |  | 16QAM | 54 | P_18@82 | S_18@82 | S_18@82 | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] clause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.1A.4.4.1-1 & 6.6.2.1A.4.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.1A.4.4.3.

6.6.2.1A.4.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels except PHICH.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.6.2.1A.4.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.6.2.1.4.1-1 on both PCC and SCC. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

6.	Measure the mean power over all component carriers in the CA configuration of the radio access mode, which shall meet the requirements described in Table 6.2.3A.1.5. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

7.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to table 6.6.2.1A.4.5 and using a rms detector, as applicable. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.6.2.1A.4.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

6.6.2.1A.4.5	Test Requirements

The measured UE mean power with the aggregated channel bandwidth as specified in clause 5.4.2A, derived in step 6, shall fulfil requirements in Table 6.2.3A.1.5 as appropriate, and the power of any UE emission shall fulfil requirements in Table.6.6.2.1A.4.5.

Table 6.6.2.1A.4.5-1: :General E-UTRA spectrum emission mask for CA Bandwidth Class C, E-UTRA bands ≤ 3GHz– intra-band contiguous CA

|  | Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 25RB+100RB (24.95 MHz) | 50RB+100RB (29.9 MHz) | 75RB+75RB (30 MHz) | 75RB+100RB (34.85 MHz) | 100RB+100RB (39.8 MHz) | Measurement bandwidth |
| 0-1 | -20.5 | -21 | -21 | -22 | -22.5 | 30 kHz |
| 1-5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | 1 MHz |
| 5-24.95 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 24.95-29.9 | -23.5 |  |  |  |  | 1 MHz |
| 29.9-29.95 |  | -23.5 |  |  |  | 1 MHz |
| 29.95-30 |  |  |  |  |  | 1 MHz |
| 30-34.85 |  |  | -23.5 |  |  | 1 MHz |
| 34.85-34.9 |  |  |  | -23.5 |  | 1 MHz |
| 34.9-35 |  |  |  |  |  | 1 MHz |
| 35-39.8 |  |  |  |  |  | 1 MHz |
| 39.8-39.85 |  |  |  |  | -23.5 | 1 MHz |
| 39.85-44.8 |  |  |  |  |  | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the aggregated channel bandwidth and below the lower edge of the aggregated channel bandwidth. |  |  |  |  |  |  |

Table 6.6.2.1A.4.5-2: General E-UTRA spectrum emission mask for CA Bandwidth Class C, 3GHz < E-UTRA bands ≤ 4.2GHz – intra-band contiguous CA

|  |  | Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) |  | 25RB+100RB (24.95 MHz) |  | 50RB+100RB (29.9 MHz) | 75RB+75RB (30 MHz) | 75RB+100RB (34.85 MHz) | 100RB+100RB (39.8 MHz) | Measurement bandwidth |
| 0-1 |  | -20.2 |  | -20.7 | -20.7 | -21.7 | -22.2 | 30 kHz |
| 1-5 |  | -8.2 |  | -8.2 | -8.2 | -8.2 | -8.2 | 1 MHz |
| 5-24.95 |  | -11.2 |  | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 24.95-29.9 |  | -23.2 |  |  |  |  |  | 1 MHz |
| 29.9-29.95 |  |  |  | -23.2 |  |  |  | 1 MHz |
| 29.95-30 |  |  |  |  |  |  |  | 1 MHz |
| 30-34.85 |  |  |  |  | -23.2 |  |  | 1 MHz |
| 34.85-34.9 |  |  |  |  |  | -23.2 |  | 1 MHz |
| 34.9-35 |  |  |  |  |  |  |  | 1 MHz |
| 35-39.8 |  |  |  |  |  |  |  | 1 MHz |
| 39.8-39.85 |  |  |  |  |  |  | -23.2 | 1 MHz |
| 39.85-44.8 |  |  |  |  |  |  |  | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the aggregated channel bandwidth and below the lower edge of the aggregated channel bandwidth. |  |  |  |  |  |  |  |  |

Table 6.6.2.1A.4.5-3: General E-UTRA spectrum emission mask for CA Bandwidth Class B, E-UTRA bands ≤ 3GHz – intra-band contiguous CA

| Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |
| --- | --- | --- | --- |
| ΔfOOB (MHz) | 25RB+50RB (14.95 MHz) | 50RB+50RB (19.9 MHz) | Measurement bandwidth |
| 0-1 | -20 | -21 | 30 kHz |
| 1-5 | -10 | -10 | 1 MHz |
| 5-14.95 | -13 | -13 | 1 MHz |
| 14.95-19.90 | -25 | -13 | 1 MHz |
| 19.90-19.95 | -25 | -25 | 1 MHz |
| 19.95-24.90 |  | -25 | 1 MHz |

Table 6.6.2.1A.4.5-4: General E-UTRA spectrum emission mask for CA Bandwidth Class B, 3GHz < E-UTRA bands ≤ 4.2GHz – intra-band contiguous CA

| Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |
| --- | --- | --- | --- |
| ΔfOOB (MHz) | 25RB+50RB (14.95 MHz) | 50RB+50RB (19.9 MHz) | Measurement bandwidth |
| 0-1 | -20 | -21 | 30 kHz |
| 1-5 | -10 | -10 | 1 MHz |
| 5-14.95 | -13 | -13 | 1 MHz |
| 14.95-19.90 | -25 | -13 | 1 MHz |
| 19.90-19.95 | -25 | -25 | 1 MHz |
| 19.95-24.90 |  | -25 | 1 MHz |

Table 6.6.2.1A.4.5-5: General E-UTRA spectrum emission mask, E-UTRA bands ≤ 3GHz – inter-band CA

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.5 | -11.5 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 1-2.5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 6-10 |  |  | -23.5 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.5 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.5 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

Table 6.6.2.1A.4.5-6: General E-UTRA spectrum emission mask, 3GHz < E-UTRA bands ≤ 4.2GHz – inter-band CA

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.2 | -11.2 | -13.2 | -16.2 | -18.2 | -19.2 | 30 kHz |
| 1-2.5 | -8.2 | -8.2 | -8.2 | -8.2 | -8.2 | -8.2 | 1 MHz |
| 2.5-2.8 | -23.2 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.2 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 6-10 |  |  | -23.2 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.2 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.2 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.2 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

##### 6.6.2.1A.5	Spectrum Emission Mask for CA (4UL)

6.6.2.1A.5.1	Test purpose

To verify that the power of any UE emission shall not exceed specified level for the specified channel bandwidth for CA.

6.6.2.1A.5.2	Test applicability

This test case applies to all types of E-UTRA UE release 11 and forward that support inter-band and 4UL CA.

6.6.2.1A.5.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.2.1A.0.

6.6.2.1A.5.4	Test description

6.6.2.1A.5.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-2. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and are shown in table 6.6.2.1A.5.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.1A.5.4.1-1: inter-band CA Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  |  |  | Low and High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE (Note 1). |  |  |  |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg DL Allocation |  |  |  |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| ID | PCC NRB | SCC1 NRB | SCC2 NRB | SCC3 NRB | PCC & SCC RB allocation |  | NRB_ alloc | PCC & SCC1 & SCC2 & SCC3 RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 100 | 50 | 100 | 50 |  | QPSK | 24 | P_12@0 | S_0@0 | S_12@0 | S_0@0 |
| 2 | 100 | 50 | 100 | 50 |  | QPSK | 100 | P_50@0 | S_0@0 | S_50@0 | S_0@0 |
| 3 | 100 | 50 | 100 | 50 |  | QPSK | 300 | P_100@0 | S_50@0 | S_100@0 | S_50@0 |
| 4 | 100 | 50 | 100 | 50 |  | 16 QAM | 24 | P_12@0 | S_0@0 | S_12@0 | S_0@0 |
| 5 | 100 | 50 | 100 | 50 |  | 16 QAM | 100 | P_50@0 | S_0@0 | S_50@0 | S_0@0 |
| 6 | 100 | 50 | 100 | 50 |  | 16 QAM | 300 | P_100@0 | S_50@0 | S_100@0 | S_50@0 |
| 7 | 100 | 50 | 100 | 50 |  | 64 QAM | 24 | P_12@0 | S_0@0 | S_12@0 | S_0@0 |
| 8 | 100 | 50 | 100 | 50 |  | 64 QAM | 300 | P_100@0 | S_50@0 | S_100@0 | S_50@0 |
| 9 | 100 | 50 | 100 | 50 | N/A for this | 256 QAM | 300 | P_100@0 | S_50@0 | S_100@0 | S_50@0 |
| 10 | 100 | 100 | 100 | 100 | test | QPSK | 24 | P_12@0 | S_0@0 | S_12@0 | S_0@0 |
| 11 | 100 | 100 | 100 | 100 |  | QPSK | 100 | P_50@0 | S_0@0 | S_50@0 | S_0@0 |
| 12 | 100 | 100 | 100 | 100 |  | QPSK | 400 | P_100@0 | S_100@0 | S_100@0 | S_100@0 |
| 13 | 100 | 100 | 100 | 100 |  | 16 QAM | 24 | P_12@0 | S_0@0 | S_12@0 | S_0@0 |
| 14 | 100 | 100 | 100 | 100 |  | 16 QAM | 100 | P_50@0 | S_0@0 | S_50@0 | S_0@0 |
| 15 | 100 | 100 | 100 | 100 |  | 16 QAM | 400 | P_100@0 | S_100@0 | S_100@0 | S_100@0 |
| 16 | 100 | 100 | 100 | 100 |  | 64 QAM | 24 | P_12@0 | S_0@0 | S_12@0 | S_0@0 |
| 17 | 100 | 100 | 100 | 100 |  | 64 QAM | 300 | P_100@0 | S_50@0 | S_100@0 | S_50@0 |
| 18 | 100 | 100 | 100 | 100 |  | 256 QAM | 400 | P_100@0 | S_100@0 | S_100@0 | S_100@0 |
| NOTE 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. NOTE 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure group A.55 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] clause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.1A.5.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.1A.5.4.3.

6.6.2.1A.5.4.2	Test procedure

1.	Configure SCCs according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels except PHICH.

2.	The SS shall configure SCCs as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.6.2.1A.5.4.3.

3.	SS activates SCCs by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.6.2.1.5.1-1 on PCC and SCCs. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

6.	Measure the mean power over all component carriers in the CA configuration of the radio access mode, which shall meet the requirements described in Table 6.2.3A.5.5-1. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

7.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to table 6.6.2.1A.5.5-1 or 6.6.2.1A.5.5-2 and using a rms detector, as applicable. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.6.2.1A.5.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

6.6.2.1A.5.5	Test Requirements

The measured UE mean power with the aggregated channel bandwidth as specified in clause 5.4.2A, derived in step 6, shall fulfil requirements in Table 6.2.3A.5.5-1 as appropriate, and the power of any UE emission shall fulfil requirements in Table.6.6.2.1A.1.5-1 or Table.6.6.2.1A.1.5-2 for Bandwidth Class C, as applicable.

Table 6.6.2.1A.5.5-1: General E-UTRA spectrum emission mask for CA Bandwidth Class C, E-UTRA bands ≤ 3GHz

|  | Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 25RB+100RB (24.95MHz) | 50RB+75RB (24.75 MHz) | 50RB+100RB (29.9 MHz) | 75RB+75RB (30 MHz) | 75RB+100RB (34.85 MHz) | 100RB+100RB (39.8 MHz) | Measurement bandwidth |
| 0-1 | -20.5 | -20.5 | -21 | -21 | -22 | -22.5 | 30 kHz |
| 1-5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | 1 MHz |
| 5-24.75 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 24.75-24.95 |  | -23.5 |  |  |  |  | 1 MHz |
| 24.95-29.75 | -23.5 |  |  |  |  |  | 1 MHz |
| 29.75-29.9 |  |  |  |  |  |  | 1 MHz |
| 29.9-29.95 |  |  | -23.5 |  |  |  | 1 MHz |
| 29.95-30 |  |  |  |  |  |  | 1 MHz |
| 30-34.85 |  |  |  | -23.5 |  |  | 1 MHz |
| 34.85-34.9 |  |  |  |  | -23.5 |  | 1 MHz |
| 34.9-35 |  |  |  |  |  |  | 1 MHz |
| 35-39.8 |  |  |  |  |  |  | 1 MHz |
| 39.8-39.85 |  |  |  |  |  | -23.5 | 1 MHz |
| 39.85-44.8 |  |  |  |  |  |  | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the aggregated channel bandwidth and below the lower edge of the aggregated channel bandwidth. |  |  |  |  |  |  |  |

Table 6.6.2.1A.5.5-2: General E-UTRA spectrum emission mask for CA Bandwidth Class C, 3GHz < E-UTRA bands ≤ 4.2GHz

|  | Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 25RB+100RB (24.95MHz) | 50RB+75RB (24.75 MHz) | 50RB+100RB (29.9 MHz) | 75RB+75RB (30 MHz) | 75RB+100RB (34.85 MHz) | 100RB+100RB (39.8 MHz) | Measurement bandwidth |
| 0-1 | -20.2 | -20.2 | -20.7 | -20.7 | -21.7 | -22.2 | 30 kHz |
| 1-5 | -8.2 | -8.2 | -8.2 | -8.2 | -8.2 | -8.2 | 1 MHz |
| 5-24.75 | -11.2 | -11.2 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 24.75-24.95 |  | -23.2 |  |  |  |  | 1 MHz |
| 24.95-29.75 | -23.2 |  |  |  |  |  | 1 MHz |
| 29.75-29.9 |  |  |  |  |  |  | 1 MHz |
| 29.9-29.95 |  |  | -23.2 |  |  |  | 1 MHz |
| 29.95-30 |  |  |  |  |  |  | 1 MHz |
| 30-34.85 |  |  |  | -23.2 |  |  | 1 MHz |
| 34.85-34.9 |  |  |  |  | -23.2 |  | 1 MHz |
| 34.9-35 |  |  |  |  |  |  | 1 MHz |
| 35-39.8 |  |  |  |  |  |  | 1 MHz |
| 39.8-39.85 |  |  |  |  |  | -23.2 | 1 MHz |
| 39.85-44.8 |  |  |  |  |  |  | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the aggregated channel bandwidth and below the lower edge of the aggregated channel bandwidth. |  |  |  |  |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

#### 6.6.2.1B	Spectrum Emission Mask for UL-MIMO

##### 6.6.2.1B.1	Test purpose

To verify that the power of any UE emission at each transmit antenna shall not exceed specified lever for the specified channel bandwidth.

##### 6.6.2.1B.2	Test applicability

This test case applies to all types of E-UTRA UE release 10 and forward that support UL-MIMO.

##### 6.6.2.1B.3	Minimum conformance requirements

For UE with multiple transmit antenna connectors, the requirements for the spectrum emission mask are specified at each transmit antenna connector. The spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the edge of the assigned E-UTRA channel bandwidth. For frequencies greater than (ΔfOOB) as specified in Table 6.6.2.1B.3-1.

For UEs with two transmit antenna connectors in closed-loop spatial multiplexing scheme , the requirements in Table 6.6.2.1B.3-1 apply to each transmit antenna connector with the UL-MIMO configurations specified in Table 6.2.2B.3-2.

Table 6.6.2.1B.3-1: General E-UTRA spectrum emission mask

| Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -10 | -13 | -15 | -18 | -20 | -21 | 30 kHz |
| 1-2.5 | -10 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.5-2.8 | -25 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.8-5 |  | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 5-6 |  | -25 | -13 | -13 | -13 | -13 | 1 MHz |
| 6-10 |  |  | -25 | -13 | -13 | -13 | 1 MHz |
| 10-15 |  |  |  | -25 | -13 | -13 | 1 MHz |
| 15-20 |  |  |  |  | -25 | -13 | 1 MHz |
| 20-25 |  |  |  |  |  | -25 | 1 MHz |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

The normative reference for this requirement is TS 36.101 clause 6.6.2B.

##### 6.6.2.1B.4	Test description

6.6.2.1B.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.1B.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2 respectively. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.1B.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] clause 4.1) |  | NC |  |  |
| Test Frequencies (as specified in TS 36.508 [7] clause 4.3.1) |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] clause 4.3.1) |  | Lowest, 5MHz, 10MHz, Highest |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Downlink Configuration |  | Uplink Configuration |  |  |
| Ch BW | N/A for SEM testing | Mod'n | RB allocation |  |
|  |  |  | FDD | TDD |
| 1.4MHz |  | QPSK | 6 | 6 |
| 1.4MHz |  | QPSK | 5 | 5 |
| 1.4MHz |  | 16QAM | 5 | 5 |
| 1.4MHz |  | 16QAM | 6 | 6 |
| 3MHz |  | QPSK | 15 | 15 |
| 3MHz |  | QPSK | 4 | 4 |
| 3MHz |  | 16QAM | 4 | 4 |
| 3MHz |  | 16QAM | 15 | 15 |
| 5MHz |  | QPSK | 25 | 25 |
| 5MHz |  | QPSK | 8 | 8 |
| 5MHz |  | 16QAM | 8 | 8 |
| 5MHz |  | 16QAM | 25 | 25 |
| 10MHz |  | QPSK | 50 | 50 |
| 10MHz |  | QPSK | 12 | 12 |
| 10MHz |  | 16QAM | 12 | 12 |
| 10MHz |  | 16QAM | 50 (Note 4) | 50 (Note 4) |
| 15MHz |  | QPSK | 75 | 75 |
| 15MHz |  | QPSK | 16 | 16 |
| 15MHz |  | 16QAM | 16 | 16 |
| 15MHz |  | 16QAM | 75 (Note 4) | 75 (Note 4) |
| 20MHz |  | QPSK | 100 | 100 |
| 20MHz |  | QPSK | 18 | 18 |
| 20MHz |  | 16QAM | 18 | 18 |
| 20MHz |  | 16QAM | 100 (Note 4) | 100 (Note 4) |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The allowed MPR for maximum output power UE might apply is described in clause 6.2.3B.3. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (max+1 - RB allocation) of the channel bandwidth. Note 4: Applies only for UE-Categories [FFS]. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure A.28.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] clause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.1B.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.1B.4.3.

6.6.2.1B.4.2	Test procedure

1.	SS sends uplink scheduling information via PDCCH DCI format 4 for C_RNTI to schedule the UL RMC according to Table 6.6.2.1B.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the sum of the mean power at each antenna connector of UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in clause 6.2.3B.5. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

4.	Measure the power of the transmitted signal at each antenna connector of UE with a measurement filter of bandwidths according to table 6.6.2.1B.5-1 or 6.6.2.1B.5-2 as applicable and using a rms detector. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.6.2.1B.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6.

##### 6.6.2.1B.5	Test requirements

The measured sum of mean power at each antenna connector of UE in the channel bandwidth, derived in step 3, shall fulfil requirements in clause 6.2.3B.5 as appropriate,

And the power of any UE emission at each transmit antenna connector shall fulfil requirements in Table.6.6.2.1B.5-1 or Table.6.6.2.1B.5-2 as applicable.

Table 6.6.2.1B.5-1: General E-UTRA spectrum emission mask, E UTRA bands ≤ 3GHz

| Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.5 | -11.5 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 1-2.5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 6-10 |  |  | -23.5 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.5 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.5 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

Table 6.6.2.1B.5-2: General E-UTRA spectrum emission mask, 3GHz < E UTRA bands ≤ 4.2GHz

| Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.2 | -11.2 | -13.2 | -16.2 | -18.2 | -19.2 | 30 kHz |
| 1-2.5 | -8.2 | -8.2 | -8.2 | -8.2 | -8.2 | -8.2 | 1 MHz |
| 2.5-2.8 | -23.2 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.2 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 6-10 |  |  | -23.2 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.2 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.2 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.2 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

#### 6.6.2.1E	Spectrum Emission Mask for UE category 0

##### 6.6.2.1E.1	Test purpose

Same test purpose as in clause 6.6.2.1.1.

##### 6.6.2.1E.2	Test applicability

This test case applies to all types of E-UTRA UE release 12 and forward of UE category 0.

##### 6.6.2.1E.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.6.2.1.3.

##### 6.6.2.1E.4	Test description

6.6.2.1E.4.1	Initial conditions

Same initial conditions as in clause 6.6.2.1.4.1 with following exceptions:

-	Instead of Table 6.6.2.1.4.1-1  use Table 6.6.2.1E.4.1-1.

-	Connect SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.3 using only main UE Tx/Rx antenna.

Table 6.6.2.1E.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  | NC |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  | Lowest, 5MHz, 10MHz, Highest |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
|  | Downlink Configuration | Uplink Configuration |  |  |
| Ch BW | N/A for SEM testing | Mod'n | RB allocation |  |
|  |  |  | FDD and HD-FDD | TDD |
| 1.4MHz |  | QPSK | 6 | 6 |
| 1.4MHz |  | QPSK | 5 | 5 |
| 1.4MHz |  | 16QAM | 5 | 5 |
| 3MHz |  | QPSK | 15 | 15 |
| 3MHz |  | QPSK | 4 | 4 |
| 3MHz |  | 16QAM | 4 | 4 |
| 3MHz |  | 16QAM | 5 | 5 |
| 5MHz |  | QPSK | 25 | 25 |
| 5MHz |  | QPSK | 8 | 8 |
| 5MHz |  | 16QAM | 5 | 5 |
| 10MHz |  | QPSK | 36 | 36 |
| 10MHz |  | QPSK | 12 | 12 |
| 10MHz |  | 16QAM | 5 | 5 |
| 15MHz |  | QPSK | 36 | 36 |
| 15MHz |  | QPSK | 16 | 16 |
| 15MHz |  | 16QAM | 5 | 5 |
| 20MHz |  | QPSK | 36 | 36 |
| 20MHz |  | QPSK | 18 | 18 |
| 20MHz |  | 16QAM | 5 | 5 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The allowed MPR for maximum output power UE might apply is described in clause 6.2.3.3. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (max+1 - RB allocation) of the channel bandwidth. |  |  |  |  |

6.6.2.1E.4.2	Test procedure

Same test procedure as in clause 6.6.2.1.4.2 with following exception:

-	Instead of Table 6.6.2.1.4.1-1  use Table 6.6.2.1E.4.1-1.

6.6.2.1E.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6.

##### 6.6.2.1E.5	Test requirements

The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.3E.5-1 as appropriate, and the power of any UE emission shall fulfil requirements in Table.6.6.2.1E.5-1 or Table.6.6.2.1E.5-2, as applicable.

Table 6.6.2.1E.5-1: General E-UTRA spectrum emission mask, E-UTRA bands ≤ 3GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.5 | -11.5 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 1-2.5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 6-10 |  |  | -23.5 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.5 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.5 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

Table 6.6.2.1E.5-2: General E-UTRA spectrum emission mask, 3GHz < E-UTRA bands ≤ 4.2GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.2 | -11.2 | -13.2 | -16.2 | -18.2 | -19.2 | 30 kHz |
| 1-2.5 | -8.2 | -8.2 | -8.2 | -8.2 | -8.2 | -8.2 | 1 MHz |
| 2.5-2.8 | -23.2 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.2 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 6-10 |  |  | -23.2 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.2 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.2 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.2 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

#### 6.6.2.1EA	Spectrum Emission Mask for UE category M1

##### 6.6.2.1EA.1	Test purpose

To verify that the power of any UE emission shall not exceed specified lever for the specified channel bandwidth.

##### 6.6.2.1EA.2	Test applicability

This test case applies to all types of E-UTRA UE release 13 and forward of UE category M1.

##### 6.6.2.1EA.3	Minimum conformance requirements

The spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the edge of the assigned E-UTRA channel bandwidth. For frequencies greater than (ΔfOOB) as specified in Table 6.6.2.1.3EA -1 the spurious requirements in clause 6.6.3EA are applicable.

The power of any UE emission shall not exceed the levels specified in Table 6.6.2.1.3EA -1 for the specified channel bandwidth.

Table 6.6.2.1EA.3-1: General E-UTRA spectrum emission mask

| Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -10 | -13 | -15 | -18 | -20 | -21 | 30 kHz |
| 1-2.5 | -10 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.5-2.8 | -25 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.8-5 |  | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 5-6 |  | -25 | -13 | -13 | -13 | -13 | 1 MHz |
| 6-10 |  |  | -25 | -13 | -13 | -13 | 1 MHz |
| 10-15 |  |  |  | -25 | -13 | -13 | 1 MHz |
| 15-20 |  |  |  |  | -25 | -13 | 1 MHz |
| 20-25 |  |  |  |  |  | -25 | 1 MHz |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

The normative reference for this requirement is TS 36.101 [2] clause 6.6.2.1.

##### 6.6.2.1EA.4	Test description

6.6.2.1EA.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in sub-clause 5.2E. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.1EA.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2 respectively. Configurations of PDSCH and MPDCCH before measurement are specified in Annex C.2.

Table 6.6.2.1EA.4.1-1: Test Configuration mask

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  | Low range, Mid range, High range |  |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  | Lowest, 5MHz, 10MHz, 15 MHz |  |  |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |  |
|  | Downlink Configuration |  | Uplink Configuration |  |  |  |
| Ch BW | N/A for Maximum Power Reduction (MPR) test case |  | Mod'n | RB allocation |  |  |
|  |  |  |  | FDD and HD-FDD | TDD | Narrowband index (Note 1) |
| Low range, Mid range |  |  |  |  |  |  |
| 1.4MHz |  |  | QPSK | 2 | 2 | 0 |
| 1.4MHz |  |  | QPSK | 5 | 5 | 0 |
| 1.4MHz |  |  | QPSK | 6 | 6 | 0 |
| 1.4MHz |  |  | 16QAM | 2 | 2 | 0 |
| 1.4MHz |  |  | 16QAM | 5 | 5 | 0 |
| 3MHz |  |  | QPSK | 2 | 2 | 0 |
| 3MHz |  |  | QPSK | 5 | 5 | 0 |
| 3MHz |  |  | QPSK | 6 | 6 | 0 |
| 3MHz |  |  | 16QAM | 2 | 2 | 0 |
| 3MHz |  |  | 16QAM | 5 | 5 | 0 |
| 5MHz |  |  | QPSK | 6 | 6 | 0 |
| 5MHz (Note 4) |  |  | 16QAM | 1 | 1 | 0 |
| 5MHz |  |  | 16QAM | 3 | 3 | 0 |
| 5MHz |  |  | 16QAM | 5 | 5 | 0 |
| 10MHz (Note 4) |  |  | QPSK | 4 | 4 | 0 |
| 10MHz |  |  | QPSK | 6 | 6 | 0 |
| 10MHz (Note 4) |  |  | 16QAM | 3 | 3 | 0 |
| 10MHz |  |  | 16QAM | 5 | 5 | 0 |
| 15MHz |  |  | QPSK | 6 | 6 | 0 |
| 15MHz |  |  | 16QAM | 5 | 5 | 0 |
| High range |  |  |  |  |  |  |
| 1.4MHz |  |  | QPSK | 2 | 2 | 0 |
| 1.4MHz |  |  | QPSK | 5 | 5 | 0 |
| 1.4MHz |  |  | QPSK | 6 | 6 | 0 |
| 1.4MHz |  |  | 16QAM | 2 | 2 | 0 |
| 1.4MHz |  |  | 16QAM | 5 | 5 | 0 |
| 3MHz |  |  | QPSK | 2 | 2 | 1 |
| 3MHz |  |  | QPSK | 5 | 5 | 1 |
| 3MHz |  |  | QPSK | 6 | 6 | 1 |
| 3MHz |  |  | 16QAM | 2 | 2 | 1 |
| 3MHz |  |  | 16QAM | 5 | 5 | 1 |
| 5MHz |  |  | QPSK | 6 | 6 | 3 |
| 5MHz (Note 4) |  |  | 16QAM | 1 | 1 | 3 |
| 5MHz |  |  | 16QAM | 3 | 3 | 3 |
| 5MHz |  |  | 16QAM | 5 | 5 | 3 |
| 10MHz (Note 4) |  |  | QPSK | 4 | 4 | 7 |
| 10MHz |  |  | QPSK | 6 | 6 | 7 |
| 10MHz (Note 4) |  |  | 16QAM | 3 | 3 | 7 |
| 10MHz |  |  | 16QAM | 5 | 5 | 7 |
| 15MHz |  |  | QPSK | 6 | 6 | 11 |
| 15MHz |  |  | 16QAM | 5 | 5 | 11 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4 Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 - RB allocation) of the narrowband. Note 4: Only for power class 3 UE. |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.1EA.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [7] clause 5.2A.2AA. Message contents are defined in clause 6.6.2.1EA.4.3.

6.6.2.1EA.4.2	Test procedure

1.	SS sends uplink scheduling information via PDCCH DCI format 6-0A for C_RNTI to schedule the UL RMC according to Table 6.6.2.1EA.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Tables 6.2.3EA.5-1 to 6.2.3EA.5-3. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test. For HD-FDD slots with transient periods are not under test. Half-duplex guard sub frame are not under test.

4.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to table 6.6.2.1EA.5-1 and using a rms detector, as applicable. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.6.2.1EA.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the condition CEModeA.

##### 6.6.2.1EA.5	Test requirements

The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Tables 6.2.3EA.5-1 to 6.2.3EA.5-7a as appropriate, and the power of any UE emission shall fulfil requirements in Table.6.6.2.1EA.5-1, as applicable.

Table 6.6.2.1EA.5-1: General E-UTRA spectrum emission mask

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.5 | -11.5 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 1-2.5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 6-10 |  |  | -23.5 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.5 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.5 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

#### 6.6.2.1EB	Spectrum Emission Mask for UE category 1bis

##### 6.6.2.1EB.1	Test purpose

Same test purpose as in clause 6.6.2.1.1.

##### 6.6.2.1EB.2	Test applicability

This test case applies to all types of E-UTRA UE release 13 and forward of UE category 1bis

##### 6.6.2.1EB.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.6.2.1.3.

##### 6.6.2.1EB.4	Test description

6.6.2.1EB.4.1	Initial conditions

Same initial conditions as in clause 6.6.2.1.4.1 with following exceptions:

-	Connect SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.3 using only main UE Tx/Rx antenna.

6.6.2.1EB.4.2	Test procedure

Same test procedure as in clause 6.6.2.1.4.2 with following exception:

-	Instead of Table 6.6.2.1.4.1-1  use Table 6.6.2.1E.4.1-1.

6.6.2.1EB.4.3	Message contents

Message contents same as in clause 6.6.2.1.4.3.

##### 6.6.2.1EB.5	Test requirements

The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.3EB.5-1 as appropriate, and the power of any UE emission shall fulfil requirements in Table.6.6.2.1EB.5-1 or Table.6.6.2.1EB.5-2, as applicable.

Table 6.6.2.1EB.5-1: General E-UTRA spectrum emission mask, E-UTRA bands ≤ 3GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.5 | -11.5 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 1-2.5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 6-10 |  |  | -23.5 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.5 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.5 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

Table 6.6.2.1EB.5-2: General E-UTRA spectrum emission mask, 3GHz < E-UTRA bands ≤ 4.2GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.2 | -11.2 | -13.2 | -16.2 | -18.2 | -19.2 | 30 kHz |
| 1-2.5 | -8.2 | -8.2 | -8.2 | -8.2 | -8.2 | -8.2 | 1 MHz |
| 2.5-2.8 | -23.2 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.2 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 6-10 |  |  | -23.2 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.2 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.2 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.2 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

#### 6.6.2.1EC	Spectrum Emission Mask for UE category M2

##### 6.6.2.1EC.1	Test purpose

To verify that the power of any UE emission shall not exceed specified lever for the specified channel bandwidth.

##### 6.6.2.1EC.2	Test applicability

This test case applies to all types of E-UTRA UE release 14 and forward of UE category M2.

##### 6.6.2.1EC.3	Minimum conformance requirements

The spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the edge of the assigned E-UTRA channel bandwidth. For frequencies greater than (ΔfOOB) as specified in Table 6.6.2.1EC.3-1, the spurious requirements in clause 6.6.3EC are applicable.

The power of any UE emission shall not exceed the levels specified in Table 6.6.2.1EC.3-1 for the specified channel bandwidth.

Table 6.6.2.1EC.3-1: General E-UTRA spectrum emission mask

| Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -10 | -13 | -15 | -18 | -20 | -21 | 30 kHz |
| 1-2.5 | -10 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.5-2.8 | -25 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.8-5 |  | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 5-6 |  | -25 | -13 | -13 | -13 | -13 | 1 MHz |
| 6-10 |  |  | -25 | -13 | -13 | -13 | 1 MHz |
| 10-15 |  |  |  | -25 | -13 | -13 | 1 MHz |
| 15-20 |  |  |  |  | -25 | -13 | 1 MHz |
| 20-25 |  |  |  |  |  | -25 | 1 MHz |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

The normative reference for this requirement is TS 36.101 [2] clause 6.6.2.1.

##### 6.6.2.1EC.4	Test description

6.6.2.1EC.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in sub-clause 5.2E. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.1EC.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2 respectively. Configurations of PDSCH and MPDCCH before measurement are specified in Annex C.2.

Table 6.6.2.1EC.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Lowest, 5MHz, 10MHz, Highest |  |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |  |
| Configuration ID | Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |  |  |
|  |  |  |  | FDD and HD-FDD | TDD | Narrowband index (Note 1) |
| Low range, Mid range |  |  |  |  |  |  |
| 1 | 1.4MHz |  | QPSK | 2 | 2 | 0 |
| 2 | 1.4MHz |  | QPSK | 5 | 5 | 0 |
| 3 | 1.4MHz |  | QPSK | 6 | 6 | 0 |
| 4 | 1.4MHz |  | 16QAM | 2 | 2 | 0 |
| 5 | 1.4MHz |  | 16QAM | 6 | 6 | 0 |
| 6 | 3MHz |  | QPSK | 2 | 2 | 0 |
| 7 | 3MHz |  | QPSK | 5 | 5 | 0 |
| 8 | 3MHz |  | QPSK | 12 | 12 | 0 |
| 9 | 3MHz |  | 16QAM | 2 | 2 | 0 |
| 10 | 3MHz |  | 16QAM | 12 | 12 | 0 |
| 11 (Note 4) | 5MHz |  | QPSK | 1 | 1 | 0 |
| 12 (Note 5) | 5MHz |  | QPSK | 3 | 3 | 0 |
| 13 | 5MHz |  | QPSK | 24 | 24 | 0 |
| 14 (Note 4) | 5MHz |  | 16QAM | 1 | 1 | 0 |
| 15 | 5MHz |  | 16QAM | 3 | 3 | 0 |
| 16 (Note 5) | 5MHz |  | 16QAM | 5 | 5 | 0 |
| 17 | 5MHz |  | 16QAM | 24 | 24 | 0 |
| 18 (Note 4) | 10MHz |  | QPSK | 4 | 4 | 0 |
| 19 (Note 5) | 10MHz |  | QPSK | 5 | 5 | 0 |
| 20 | 10MHz |  | QPSK | 24 | 24 | 0 |
| 21 (Note 4) | 10MHz |  | 16QAM | 3 | 3 | 0 |
| 22 | 10MHz |  | 16QAM | 5 | 5 | 0 |
| 23 | 10MHz |  | 16QAM | 24 | 24 | 0 |
| 24 | 15MHz |  | QPSK | 16 | 16 | 0 |
| 25 | 15MHz |  | QPSK | 24 | 24 | 0 |
| 26 | 15MHz |  | 16QAM | 16 | 16 | 0 |
| 27 | 15MHz |  | 16QAM | 24 | 24 | 0 |
| 28 | 20MHz |  | QPSK | 18 | 18 | 0 |
| 29 | 20MHz |  | QPSK | 24 | 24 | 0 |
| 30 | 20MHz |  | 16QAM | 18 | 18 | 0 |
| 31 | 20MHz |  | 16QAM | 24 | 24 | 0 |
| High range |  |  |  |  |  |  |
| 1 | 1.4MHz |  | QPSK | 2 | 2 | 0 |
| 2 | 1.4MHz |  | QPSK | 5 | 5 | 0 |
| 3 | 1.4MHz |  | QPSK | 6 | 6 | 0 |
| 4 | 1.4MHz |  | 16QAM | 2 | 2 | 0 |
| 5 | 1.4MHz |  | 16QAM | 6 | 6 | 0 |
| 6 | 3MHz |  | QPSK | 2 | 2 | 1 |
| 7 | 3MHz |  | QPSK | 5 | 5 | 1 |
| 8 | 3MHz |  | QPSK | 12 | 12 | 0 |
| 9 | 3MHz |  | 16QAM | 2 | 2 | 1 |
| 10 | 3MHz |  | 16QAM | 12 | 12 | 0 |
| 11 (Note 4) | 5MHz |  | QPSK | 1 | 1 | 3 |
| 12 (Note 5) | 5MHz |  | QPSK | 3 | 3 | 3 |
| 13 | 5MHz |  | QPSK | 24 | 24 | 0 |
| 14 (Note 4) | 5MHz |  | 16QAM | 1 | 1 | 3 |
| 15 | 5MHz |  | 16QAM | 3 | 3 | 3 |
| 16 (Note 5) | 5MHz |  | 16QAM | 5 | 5 | 3 |
| 17 | 5MHz |  | 16QAM | 24 | 24 | 0 |
| 18 (Note 4) | 10MHz |  | QPSK | 4 | 4 | 7 |
| 19 (Note 5) | 10MHz |  | QPSK | 5 | 5 | 7 |
| 20 | 10MHz |  | QPSK | 24 | 24 | 4 |
| 21 (Note 4) | 10MHz |  | 16QAM | 3 | 3 | 7 |
| 22 | 10MHz |  | 16QAM | 5 | 5 | 7 |
| 23 | 10MHz |  | 16QAM | 24 | 24 | 4 |
| 24 | 15MHz |  | QPSK | 15 | 15 | 9 |
| 25 | 15MHz |  | QPSK | 24 | 24 | 8 |
| 26 | 15MHz |  | 16QAM | 15 | 15 | 9 |
| 27 | 15MHz |  | 16QAM | 24 | 24 | 8 |
| 28 | 20MHz |  | QPSK | 18 | 18 | 13 |
| 29 | 20MHz |  | QPSK | 24 | 24 | 12 |
| 30 | 20MHz |  | 16QAM | 18 | 18 | 13 |
| 31 | 20MHz |  | 16QAM | 24 | 24 | 12 |
| Note 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.7. Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 – (RB allocation mod 6)) of the narrowband. Note 4: Only for power class 3 UE Note 5: Only for power class 5 or power class 6 UE |  |  |  |  |  |  |

Table 6.6.2.1EC.4.1-2: Test Configuration Table, subPRB allocation

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Lowest, 5MHz, 10MHz, 15 MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| Low range, Mid range |  |  |  |  |  |  |
| 1 | 5MHz |  | π/2-BPSK | ¼ | 0 |  |
| 2 | 5MHz |  | QPSK | ¼ | 0 |  |
| 3 | 5MHz |  | QPSK | ½ | 0 |  |
| 4 | 10MHz |  | π/2-BPSK | ¼ | 0 |  |
| 5 | 10MHz |  | π/2-BPSK | ¼ | 4 |  |
| 6 | 10MHz |  | QPSK | ¼ | 0 |  |
| 7 | 10MHz |  | QPSK | ¼ | 4 |  |
| 8 | 10MHz |  | QPSK | ½ | 0 |  |
| 9 | 10MHz |  | QPSK | ½ | 4 |  |
| 10 | 15MHz |  | π/2-BPSK | ¼ | 0 |  |
| 11 | 15MHz |  | π/2-BPSK | ¼ | 8 |  |
| 12 | 15MHz |  | QPSK | ¼ | 0 |  |
| 13 | 15MHz |  | QPSK | ¼ | 8 |  |
| 14 | 15MHz |  | QPSK | ½ | 0 |  |
| 15 | 15MHz |  | QPSK | ½ | 8 |  |
| High range |  |  |  |  |  |  |
| 16 | 5MHz |  | π/2-BPSK | ¼ | 0 |  |
| 17 | 5MHz |  | QPSK | ¼ | 0 |  |
| 18 | 5MHz |  | QPSK | ½ | 0 |  |
| 19 | 10MHz |  | π/2-BPSK | ¼ | 0 |  |
| 20 | 10MHz |  | π/2-BPSK | ¼ | 4 |  |
| 21 | 10MHz |  | QPSK | ¼ | 0 |  |
| 22 | 10MHz |  | QPSK | ½ | 4 |  |
| 23 | 10MHz |  | QPSK | ¼ | 0 |  |
| 24 | 10MHz |  | QPSK | ½ | 4 |  |
| 25 | 15MHz |  | π/2-BPSK | ¼ | 0 |  |
| 26 | 15MHz |  | π/2-BPSK | ¼ | 8 |  |
| 27 | 15MHz |  | QPSK | ¼ | 0 |  |
| 28 | 15MHz |  | QPSK | ¼ | 8 |  |
| 29 | 15MHz |  | QPSK | ½ | 0 |  |
| 30 | 15MHz |  | QPSK | ½ | 8 |  |
| Note 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.7. Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 – (RB allocation mod 6)) of the narrowband. |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.1EC.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [7] clause 5.2A.2AA. Message contents are defined in clause 6.6.2.1EC.4.3.

6.6.2.1EC.4.2	Test procedure

1.	SS sends uplink scheduling information via PDCCH DCI format 6-0A for C_RNTI to schedule the UL RMC according to Table 6.6.2.1EC.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Tables 6.2.3EC.5-1 to 6.2.3EC.5-3. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test. For HD-FDD slots with transient periods are not under test. Half-duplex guard sub frame are not under test.

4.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to table 6.6.2.1EC.5-1 and using a rms detector, as applicable. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

5.	For UE supporting subPRB allocation, repeat step 1-4 with UL RMC according to Table 6.6.2.1EC.4.1-2.

6.6.2.1EC.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the condition CEModeA.

##### 6.6.2.1EC.5	Test requirements

The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Tables 6.2.3EC.5-1 to 6.2.3EC.5-3 as appropriate, and the power of any UE emission shall fulfil requirements in Table.6.6.2.1EC.5-1, as applicable.

Table 6.6.2.1EC.5-1: General E-UTRA spectrum emission mask

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.5 | -11.5 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 1-2.5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 6-10 |  |  | -23.5 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.5 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.5 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth

#### 6.6.2.1F	Spectrum Emission Mask for category NB1 and NB2

6.6.2.1F.1	Test purpose

To verify that the power of the category NB1 and NB2 UE emission shall not exceed specified level for the specified channel bandwidth.

6.6.2.1F.2	Test applicability

This test case applies to all types of E-UTRA FDD UE release 13 and forward of category NB1.

This test case applies to all types of E-UTRA FDD UE release 14 and forward of category NB2.

This test case applies to all types of E-UTRA TDD UE release 15 and forward of category NB1 and NB2.

6.6.2.1F.3	Minimum conformance requirements

The spectrum emission mask of the category NB1 and NB2 UE applies to frequencies (ΔfOOB) starting from the  edge of the assigned category NB1 or NB2 channel bandwidth. For frequencies greater than (ΔfOOB) as specified in Table 6.6.2.1F.3-1 the spurious requirements in subclause 6.6.3 are applicable.

The power of any category NB1 or NB2 UE emission shall not exceed the levels specified in Table 6.6.2.1F.3-1.

Table 6.6.2.1F.3-1: category NB1 and NB2 UE spectrum emission mask

| ΔfOOB (kHz) | Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 0 | 26 | 30 kHz |
| 100 | -5 | 30 kHz |
| 150 | -8 | 30 kHz |
| 300 | -29 | 30 kHz |
| 500-1700 | -35 | 30 kHz |

In addition to the spectrum emission mask requirement in Table 6.6.2.1F.3-1 a category NB1 or NB2 UE shall also meet the applicable E-UTRA spectrum emission mask requirement in sub-clause 6.6.2. E-UTRA spectrum emission requirement applies for frequencies that are Foffset away from edge of NB1 or NB2 channel edge as defined in Table 6.6.2.1F.3-2.

Table 6.6.2.1F.3-2: Foffset for category NB1 and NB2 UE spectrum emission mask

| Channel BW (MHz) | Foffset [kHz] |
| --- | --- |
| 1.4 | 165 |
| 3 | 190 |
| 5 | 200 |
| 10 | 225 |
| 15 | 240 |
| 20 | 245 |

NOTE: Foffset in Table 6.6.2.1F.3-2 is used to guarantee co-existence for guard-band operation.

The normative reference for this requirement is TS 36.101 [2] clause 6.6.2F.1.

##### 6.6.2.1F.4	Test description

6.6.2.1F.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions and test frequencies based on the subset of E-UTRA operating bands defined for NB-IoT in clause 5.2F. All of these configurations shall be tested with applicable test parameters shown in table 6.6.2.1F.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of NPDSCH and NPDCCH before measurement are specified in Annex C.2.

Table 6.6.2.1F.4.1-1: Test Configuration Table for FDD & TDD

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 8.1.1 |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 8.1.3.1 |  | Frequency ranges defined in Annex K.1.2 |  |  |
| Test Parameters |  |  |  |  |
| Configuration ID | Downlink Configuration | Uplink Configuration |  |  |
|  | N/A | Modulation | Ntones | Sub-carrier spacing (kHz) |
| 1 |  | QPSK | 1@0 | 3.75kHz |
| 2 |  | QPSK | 1@47 | 3.75kHz |
| 3 |  | QPSK | 1@0 | 15kHz |
| 4 |  | QPSK | 1@11 | 15kHz |
| 5 (Note 1) |  | QPSK | 3@0 | 15kHz |
| 6 (Note 1) |  | QPSK | 3@3 | 15kHz |
| 7 (Note 1) |  | QPSK | 3@9 | 15kHz |
| 8 (Note 1) |  | QPSK | 6@0 | 15kHz |
| 9 (Note 1) |  | QPSK | 6@6 | 15kHz |
| 10 (Note 1) |  | QPSK | 12@0 | 15kHz |
| Note 1: The allowed MPR for maximum output power UE might apply is described in clause [6.2.3F.3]. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure A.3 using only main Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1.1 and H.4.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.1F.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5. Message contents are defined in clause 6.6.2.1F.4.3.

6.6.2.1F.4.2	Test procedure

1.	SS sends uplink scheduling information via NPDCCH DCI format N0 for C_RNTI to schedule the UL RMC according to Table 6.6.2.1F.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC. (UE should be already transmitting PUMAX after Initial Conditions setting).

2.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Table 6.2.3F.5-1. The measurement duration is at least one sub-frame (1ms) for 15 KHz channel spacing, and at least a 2ms slot (excluding the 2304Ts gap when UE is not transmitting) respectively for the 3.75 KHz channel spacing.

3.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to table 6.6.2.1F.5-1 as applicable and using a rms detector. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs. For TDD slots with transient periods are not under test.

NOTE 1:	For configuration IDs applicable to UE depending on UE capability in Test Configuration Table with different UL sub-carrier spacing, the SS shall release the connection through State 3A-NB and finally ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5 using the appropriate UL subcarrier spacing in Random Access Response message.

6.6.2.1F.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 8.1.6.

##### 6.6.2.1F.5	Test requirements

The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.3F.5-1 as appropriate, and the power of any UE emission shall fulfil requirements in 6.6.2.1F.5-1 as applicable.

Table 6.6.2.1F.5-1: category NB1 and NB2 UE spectrum emission mask, category NB1/NB2 bands ≤ 3GHz

| ΔfOOB (kHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 0 -100 | (27.5+(F-0)×-3.5-27.5100-0) | 30 kHz |
| 100 - 150 | (-3.5+(F-100)×-6.5-(-3.5)150-100) | 30 kHz |
| 150 - 300 | (-6.5+(F-150)×-27.5-(-6.5)300-150) | 30 kHz |
| 300 - 500 | (-27.5+(F-300)×-33.5-(-27.5)500-300) | 30 kHz |
| 500 - 1700 | -33.5 | 30 kHz |
| Note 1: The limit shall be calculated for the measurement frequency F given in kHz, cantered in the measurement bandwidth. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 30 kHz filter is the inside of +15 kHz and -15 kHz, respectively. The filter shall be stepped to cover the whole range. |  |  |

Table 6.6.2.1F.5-2: Void

NOTE 1:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

NOTE 2:	The minimum requirements in clause 6.6.2.1F.3 have been combined into table 6.6.2.1F.5-1 to reduce testing complexity and test time.

#### 6.6.2.1G	Spectrum Emission Mask for V2X Communication

##### 6.6.2.1G.1	Spectrum Emission Mask for V2X Communication Non-concurrent with E-UTRA uplink transmissions

###### 6.6.2.1G.1.1	Test purpose

To verify that the power of any UE emission shall not exceed specified level for the specified channel bandwidth when UE is configured for E-UTRA V2X sidelink transmissions non-concurrent with E-UTRA uplink transmissions for E-UTRA V2X operating bands.

###### 6.6.2.1G.1.2	Test applicability

This test case applies to UE which support V2X Sidelink communication and Band 47.

###### 6.6.2.1G.1.3	Minimum conformance requirements

When UE is configured for E-UTRA V2X sidelink transmissions non-concurrent with E-UTRA uplink transmissions for E-UTRA V2X operating bands specified in TS 36.101 [2] Table 5.5G-1, the requirements in TS 36.101[2] subclause 6.6.2 apply except for the ACLR requirements for power class 2 V2X UE.

The spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the edge of the assigned E-UTRA channel bandwidth. The power of any UE emission shall not exceed the levels specified in Table 6.6.2.1G.1.3-1.

Table 6.6.2.1G.1.3-1: General E-UTRA spectrum emission mask

| Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -10 | -13 | -15 | -18 | -20 | -21 | 30 kHz |
| 1-2.5 | -10 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.5-2.8 | -25 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.8-5 |  | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 5-6 |  | -25 | -13 | -13 | -13 | -13 | 1 MHz |
| 6-10 |  |  | -25 | -13 | -13 | -13 | 1 MHz |
| 10-15 |  |  |  | -25 | -13 | -13 | 1 MHz |
| 15-20 |  |  |  |  | -25 | -13 | 1 MHz |
| 20-25 |  |  |  |  |  | -25 | 1 MHz |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

The normative reference for this requirement is TS 36.101 [2] clause 6.6.2G.

###### 6.6.2.1G.1.4	Test description

6.6.2.1G.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in sub-clause 5.2G. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.1G.1.4.1-1 the details of the V2X reference measurement channels (RMCs) are specified in sub-clause A.8.3 and the GNSS configuration in TS 36.508 [7] subclause 4.11.

Table 6.6.2.1G.1.4.1-1: Test Configuration for V2X sidelink transmission

| Initial Conditions |  |  |
| --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  | NC |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  | Low range, Mid range, High range |
| Test Channel Bandwidths (as specified in subclause 5.4.2G) |  | Lowest, Highest |
| Test Parameters for Channel Bandwidths |  |  |
|  | V2X Configuration to Trasmit |  |
| Ch BW | Mod'n | PSSCH RB Allocation |
| 10MHz | QPSK | 12 (Note 3) |
| 10MHz | QPSK | 48@2 |
| 10MHz | 16QAM | 12 (Note 3) |
| 10MHz | 16QAM | 48@2 |
| 20MHz | QPSK | 18 (Note 3) |
| 20MHz | QPSK | 96@2 |
| 20MHz | 16QAM | 18 (Note 3) |
| 20MHz | 16QAM | 96@2 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in specified in subclause 5.4.2G. Note 2: The allowed MPR for maximum output power UE might apply is described in clause 36.101[2] subclause 6.2.3G. Note 3: The test shall be performed with allocated sub-channel(s) at low and high end of the channel bandwidth respectively. |  |  |

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 36.508 [7] Figure A.92.

2.	The parameter settings for the V2X sidelink transmission over PC5 for both SS and UE are pre-configured according to TS 36.508 [7] subclause 4.10.1. Message content exceptions are defined in clause 6.6.2.1G.1.4.3.

3.	The V2X reference measurement channel is set according to Table 6.6.2.1G.1.4.1-1.

4.	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS36.508 [7] Table 4.11.2-2. Geographical area #1 is also pre-configured in the UE

5.	Propagation conditions are set according to Annex B.0;

6.	Ensure the UE is in State 5A-V2X  in Transmit Mode according to TS 36.508 [7] clause 4.5.9 , Message contents are defined in clause 6.6.2.1G.1.4.3

7.	The GNSS simulator is triggered to start step 1 of Scenario #1 to simulate a location in the centre of Geographical area #1. Wait for the UE to acquire the GNSS signal and start to transmit.

6.6.2.1G.1.4.2	Test procedure

1.	The V2X UE schedules the V2X RMC with transmission power at PUMAX level according to SL-V2X-Preconfiguration which is in line with the test configuration in Table 6.6.2.1G.1.4.1-1;

2.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Table 6.2.3G.1.1.5-1for Power Class 3 UEs or in [6.2.3G.2.1.5-1] for Power Class 2 UEs. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms).

3.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to table 6.6.2.1G.1.5-1 and using a rms detector. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.6.2.1G.1.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with following exceptions listed in the Table 6.6.2.1G.1.4.3-1 and Table 6.6.2.1G.1.4.3-2.

Table 6.6.2.1G.1.4.3-1: SL-V2X-Preconfiguration

| Derivation Path: 36.331 clause 9.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-V2X-Preconfiguration-r14 ::= SEQUENCE { |  |  |  |
| v2x-PreconfigFreqList-r14 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| v2x-CommPreconfigGeneral-r14 SEQUENCE { |  | SL-PreconfigGeneral-r12 |  |
| maxTxPower-r12 | 23 |  | Power class3 |
|  | 26 |  | Power class2 |
| sl-bandwidth-r12 | n50 | 10Mhz |  |
|  | n100 | 20Mhz |  |
| } |  |  |  |
| } |  |  |  |
| v2x-CommPreconfigSync-r14 SEQUENCE { |  | SL-PreconfigSync-r12 |  |
| syncOffsetIndicators-r14 SEQUENCE { |  |  |  |
| syncOffsetIndicator1-r14 | Not Present | Double synchronization signal transmission |  |
| } |  |  |  |
| } |  |  |  |
| v2x-CommRxPoolList-r14 SEQUENCE (SIZE (1..maxSL-V2X-RxPoolPreconf-r14)) OF SL-V2X-PreconfigCommPool-r14 { | 1 entry |  |  |
| SL-V2X-PreconfigCommPool-r14[1] | SL-CommResourcePoolV2X-r14-DEFAULT with exception listed in Table 6.6.2.1G.1.4.3-2 |  |  |
| } |  |  |  |
| v2x-CommTxPoolList-r14 SEQUENCE (SIZE (1..maxSL-V2X-TxPoolPreconf-r14)) OF SL-V2X-PreconfigCommPool-r14 { |  |  |  |
| SL-V2X-PreconfigCommPool-r14[1] | SL-V2X-PreconfigCommPool-r14-DEFAULT with exception listed in Table 6.6.2.1G.1.4.3-2 |  |  |
| } |  |  |  |
| v2x-ResourceSelectionConfig | SL-CommTxPoolSensingConfig-r14-DEFAULT with exception listed in Table 6.6.2.1G.1.4.3-3 |  |  |
| } |  |  |  |

Table 6.6.2.1G.1.4.3-2: SL-V2X-PreconfigCommPool-r14-DEFAULT

| Derivation Path: 36.331 clause 6.3.8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-CommResroucePoolV2X-r14-DEFAULT ::= SEQUENCE { |  |  |  |
| sl-Subframe-r14 CHOICE { |  |  |  |
| bs20-r14 | 11111111111111111111 |  | BITMAP_6 |
| } |  |  |  |
| sizeSubchannel-r14 | n15 |  | BW10 |
|  | n50 |  | BW10 |
|  | n20 |  | BW20 |
|  | n100 |  | BW20 |
| dataTxParameters-r14 SEQUENCE { |  |  |  |
| p0-r12 | 31 | It’s set to the maximum value to disable its effect on transmission power. |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.2.1G.1.4.3-2: SL-PSSCH-TxParameters-r14

| Derivation Path: 36.331 clause 6.3.8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-PSSCH-TxParameters-r14 ::= SEQUENCE { |  |  |  |
| minMCS-PSSCH-r14 | 0 |  | QPSK |
|  | 11 |  | 16QAM |
| maxMCS-PSSCH-r14 | 10 |  | QPSK |
|  | 19 |  | 16QAM |
| } |  |  |  |

###### 6.6.2.1G.1.5	Test requirements

The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.3G.1.1.5-1as appropriate, and the power of any UE emission shall fulfil requirements in Table.6.6.2.1G.1.5-1.

Table 6.6.2.1G.1.5-1: General E-UTRA spectrum emission mask, E-UTRA bands ≥ 5GHz

| Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |
| --- | --- | --- | --- |
| ΔfOOB (MHz) | 10 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -16 | -19 | 30 kHz |
| 1-2.5 | -8 | -8 | 1 MHz |
| 2.5-2.8 | -8 | -8 | 1 MHz |
| 2.8-5 | -8 | -8 | 1 MHz |
| 5-6 | -12 | -11 | 1 MHz |
| 6-10 | -12 | -11 | 1 MHz |
| 10-15 | -23 | -11 | 1 MHz |
| 15-20 |  | -11 | 1 MHz |
| 20-25 |  | -23 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

##### 6.6.2.1G.2	Spectrum Emission Mask for V2X Communication /Sidelink simultaneous with E-UTRA uplink transmissions

###### 6.6.2.1G.2.1	Test purpose

To verify that the power of any UE emission shall not exceed specified level for the specified channel bandwidth when UE is configured for configured for simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions for inter-band E-UTRA V2X / E-UTRA band

###### 6.6.2.1G.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 14 and forward that supports V2X communication and is capable of V2X Sidelink communication simultaneous with E-UTRA uplink transmission.

###### 6.6.2.1G.2.3	Minimum conformance requirements

When UE is configured for simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions for inter-band E-UTRA V2X / E-UTRA bands specified in TS 36.101 [2] Table 5.5G-2, the requirements in TS 36.101 [2] subclause 6.6.2 apply per V2X sidelink transmission and E-UTRA uplink transmission as specified for the corresponding inter-band con-current operation with uplink assigned to two bands.

Consequently, the emission power of the V2X UE per sidelink and E-UTRA link shall not exceed the values specified in Tables 6.6.2.1G.2.2-1

Table 6.6.2.1G.2.2-1 General E-UTRA spectrum emission mask

| Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -10 | -13 | -15 | -18 | -20 | -21 | 30 kHz |
| 1-2.5 | -10 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.5-2.8 | -25 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.8-5 |  | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 5-6 |  | -25 | -13 | -13 | -13 | -13 | 1 MHz |
| 6-10 |  |  | -25 | -13 | -13 | -13 | 1 MHz |
| 10-15 |  |  |  | -25 | -13 | -13 | 1 MHz |
| 15-20 |  |  |  |  | -25 | -13 | 1 MHz |
| 20-25 |  |  |  |  |  | -25 | 1 MHz |

The normative reference for this requirement is TS 36.101 [2] subclause 6.6.2G and subclause 6.6.2.

###### 6.6.2.1G.2.4	Test description

6.6.2.1G.2.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on Inter-band con-current V2X configurations specified in table 5.4.2G.1-2. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.1G.2.1.4.1-1. The details of the uplink and V2X reference measurement channels (RMCs) are specified in sub-clause A.2 and A.8.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.1G.2.4.1-1: Test Configuration

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  |  | Low range for uplink and Low range for V2X, High range for uplink and High range for V2X |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  |  | Lowest NRB_agg, Highest NRB_agg |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |  |
|  | Downlink | Uplink Configuration |  |  | V2X Configuration |  |  |
| Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |  | Ch BW | Mod'n | PSSCH RB allocation |
| 1.4MHz |  | QPSK | 5 |  | 10 MHz | QPSK | 48@2 |
| 1.4MHz |  | QPSK | 6 |  | 10 MHz | 16QAM | 48@2 |
| 1.4MHz |  | 16QAM | 5 |  | 20 MHz | QPSK | 96@2 |
| 1.4MHz |  | 16QAM | 6 |  | 20 MHz | 16QAM | 96@2 |
| 3MHz |  | QPSK | 4 |  |  |  |  |
| 3MHz |  | QPSK | 15 |  |  |  |  |
| 3MHz |  | 16QAM | 4 |  |  |  |  |
| 3MHz |  | 16QAM | 15 |  |  |  |  |
| 5MHz |  | QPSK | 8 |  |  |  |  |
| 5MHz |  | QPSK | 25 |  |  |  |  |
| 5MHz |  | 16QAM | 8 |  |  |  |  |
| 5MHz |  | 16QAM | 25 |  |  |  |  |
| 10MHz |  | QPSK | 12 |  |  |  |  |
| 10MHz |  | QPSK | 50 |  |  |  |  |
| 10MHz |  | 16QAM | 12 |  |  |  |  |
| 10MHz |  | 16QAM | 50 (Note 3) |  |  |  |  |
| 15MHz |  | QPSK | 16 |  |  |  |  |
| 15MHz |  | QPSK | 75 |  |  |  |  |
| 15MHz |  | 16QAM | 16 |  |  |  |  |
| 15MHz |  | 16QAM | 75 (Note 3) |  |  |  |  |
| 20MHz |  | QPSK | 18 |  |  |  |  |
| 20MHz |  | QPSK | 100 |  |  |  |  |
| 20MHz |  | 16QAM | 18 |  |  |  |  |
| 20MHz |  | 16QAM | 100 (Note 3) |  |  |  |  |
| Note 1: V2X con-current band configurations are checked separately for each band configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2G.1-2. Note 2: For Low range the E-UTRA carrier uses only test points with RBstart=0. For High range the E-UTRA carrier uses only test points with RBstart=RBmax-LCRB+1. |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.93.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3. Message content exceptions are defined in clause 6.6.2.1G.2.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL and V2X Reference Measurement Channels are set according to table 6.6.2.1G.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF-V2X according to TS 36.508 [7] clause 5.2A.2C. Message content exceptions are defined in clause 6.2.3G.1.3.4.3.

6.6.2.1G.2.4.2	Test procedure

SS sends RRC reconfiguration message specified in clause 6.6.2.1G.2.4.3 to enable PSCCH and PSSCH transmission at the PUMAX level.;

1.	The SS simultaneously sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC and V2X scheduling information via PDCCH DCI format 5A for SL_V_RNTI to schedule the V2X RMC according to Table 6.6.2.1G.2.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL and V2X RMCs.

2.	SS sends continuously uplink power control "up" commands in the uplink scheduling information to the UE to ensure that the UE transmits at PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Table 6.2.3G.1.3.5-1. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). On E-UTRA carrier, the TDD slots with transient periods are not under test.

4.	Measure the power of the transmitted signal for E-UTRA uplink with a measurement filter of bandwidths according to table 6.6.2.1G.2.5-1or 6.6.2.1G.2.5-2 and using a rms detector, as applicable. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs. On E-UTRA carrier, the TDD slots with transient periods are not under test.

5.	Measure the power of the transmitted signal for V2X sidelink with a measurement filter of bandwidths according to table 6.6.2.1G.2.5-3 and using a rms detector, as applicable. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.6.2.1G.2.4.3	Message content

Message contents are according to TS 36.508 [7] subclause 4.6.

###### 6.6.2.1G.2.5	Test requirements

The measured UE mean power in the channel bandwidth, derived in step 3 and step 4 shall fulfil requirements in Table 6.2.3G.1.3.5-1, and the power of any UE emission shall fulfil requirements in Table 6.6.2.1G.1.5-1~ Table 6.6.2.1G.2.5-3.

Table 6.6.2.1G.2.5-1: General E-UTRA spectrum emission mask, E-UTRA bands ≤ 3GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.5 | -11.5 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 1-2.5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 6-10 |  |  | -23.5 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.5 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.5 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

Table 6.6.2.1G.2.5-2: General E-UTRA spectrum emission mask, 3GHz < E-UTRA bands ≤ 4.2GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.2 | -11.2 | -13.2 | -16.2 | -18.2 | -19.2 | 30 kHz |
| 1-2.5 | -8.2 | -8.2 | -8.2 | -8.2 | -8.2 | -8.2 | 1 MHz |
| 2.5-2.8 | -23.2 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.2 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 6-10 |  |  | -23.2 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.2 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.2 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.2 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

Table 6.6.2.1G.2.5-3: General E-UTRA V2X spectrum emission mask, E-UTRA bands ≥ 5GHz

| Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |
| --- | --- | --- | --- |
| ΔfOOB (MHz) | 10 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -16 | -19 | 30 kHz |
| 1-2.5 | -8 | -8 | 1 MHz |
| 2.5-2.8 | -8 | -8 | 1 MHz |
| 2.8-5 | -8 | -8 | 1 MHz |
| 5-6 | -12 | -11 | 1 MHz |
| 6-10 | -12 | -11 | 1 MHz |
| 10-15 | -23 | -11 | 1 MHz |
| 15-20 |  | -11 | 1 MHz |
| 20-25 |  | -23 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

##### 6.6.2.1G.3	Spectrum Emission Mask for V2X Communication / Intra-band contiguous MCC operation

###### 6.6.2.1G.3.1	Test purpose

To verify that the power of any UE emission shall not exceed specified level for the specified channel bandwidth when UE is configured for Intra-band contiguous MCC operation V2X sidelink transmissions.

###### 6.6.2.1G.3.2	Test Applicability

This test case applies to UE which support Intra-band contiguous multi-carrier operation V2X Sidelink communication and Band 47.

###### 6.6.2.1G.3.3	Minimum conformance requirements

For intra-band contiguous multi-carrier operation, the general CA spectrum emission mask for Bandwidth Class B shall apply, which refers to the minimum conformance requirements defined in clause 6.6.2.1A.0.

The normative reference for this requirement is TS 36.101 [2] clause 6.6.2G.

###### 6.6.2.1G.3.4	Test Description

###### 6.6.2.1G.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in sub-clause 5.2G. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.1G.3.4.1-1 the details of the V2X reference measurement channels (RMCs) are specified in sub-clause A.8.3 and the GNSS configuration in TS 36.508 [7] subclause 4.11.

Table 6.6.2.1G.3.4.1-1: Test Configuration Table for PSSCH and PSCCH

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range , High range |  |
| Test CC Combination setting(NRB_agg) as specified in subclause 5.4.2G |  |  | Lowest NRB_agg, Highest NRB_agg |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| V2X Multi-carrier Configuration |  | V2X PSSCH Allocation |  |  |
| CC1 NRB | CC2 NRB | CC1 & CC2 Mod |  | CC1 and CC2 RB Allocation |
| 50 | 50 | QPSK |  | 12@2+0@0 |
| 50 | 50 | QPSK |  | 0@0+12@37 |
| 50 | 50 | QPSK |  | 48@2+0@0 |
| 50 | 50 | QPSK |  | 0@0+48@2 |
| 50 | 50 | QPSK |  | 48@2+48@2 |
| 50 | 50 | 16QAM |  | 12@2+0@0 |
| 50 | 50 | 16QAM |  | 0@0+12@37 |
| 50 | 50 | 16QAM |  | 48@2+0@0 |
| 50 | 50 | 16QAM |  | 0@0+48@2 |
| 50 | 50 | 16QAM |  | 48@2+48@2 |
| 50 | 50 | QPSK |  | 1@2+1@47 |

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 36.508 [7] Figure A.89a.

2.	Propagation conditions are set according to Annex B.0.

2a	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS36.508 [7] Table 4.11.2-2. Geographical area #1 is also pre-configured in the UE.

3.	Ensure the UE is in test state 5A-V2X, according to TS 36.508 [7] clause 4.5.9. Message contents are defined in clause 6.6.2.1G.3.4.3.

4.	The GNSS simulator is triggered to start step 1 of Scenario #1 to simulate a location in the centre of Geographical area #1. Wait for the UE to acquire the GNSS signal and start to transmit.

###### 6.6.2.1G.3.4.2	Test procedure

1.	Set the V2X sidelink communication parameters for both the V2X sidelink capable UE (hereinafter referred to as V2X UE) and SS according to SL-V2X-Preconfiguration in TS36.508[7] Table subclause 4.10  with the exception specified in table 6.6.2.1G.3.4.3-1;

2.	The V2X UE schedules the V2X RMC with transmission power at PUMAX level according to SL-V2X-Preconfiguration which is in line with the test configuration in Table 6.6.2.1G.3.4.1-1;

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Table 6.2.3G.1.1.5-1 for Power Class 3 . The period of the measurement shall be at least the continuous duration of one sub-frame (1ms).

4.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to table 6.6.2.1G.3.5-1 and using a rms detector. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.6.2.1G.3.4.3	Message Contents

Message content are according to 36.508[7] subclause 4.10 Table 4.10.1.1-2 with the exception listed in table 6.6.2.1G.3.4.3-1, 6.6.2.1G.3.4.3-2 and 6.6.2.1G.3.4.3-3

Table 6.6.2.1G.3.4.3-1: SL-V2X-PreconfigFreqInfo-r14-DEFAULT for CC1 & CC2

| Derivation Path: 36.331 clause 9.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-V2X-PreconfigFreqInfo-r14-DEFAULT ::= SEQUENCE { |  |  |  |
| v2x-CommPreconfigGeneral-r14 SEQUENCE { |  | SL-PreconfigGeneral-r12 |  |
| maxTxPower-r12 | 23 |  | Power class 3 |
| sl-bandwidth-r12 | n50 | 10Mhz | V2X_47B |
| } |  |  |  |
| } |  |  |  |
| v2x-CommRxPoolList-r14 SEQUENCE (SIZE (1..maxSL-V2X-RxPoolPreconf-r14)) OF SL-V2X-PreconfigCommPool-r14 { | 1 entry |  |  |
| SL-V2X-PreconfigCommPool-r14[1] | SL-V2X-PreconfigCommPool-r14-DEFAULT using condition BITMAP_6 and exception listed in Table 6.6.2.1G.3.4.3-2 | SL-V2X-PreconfigCommPool-r14 |  |
| } |  |  |  |
| v2x-CommTxPoolList-r14 SEQUENCE (SIZE (1..maxSL-V2X-TxPoolPreconf-r14)) OF SL-V2X-PreconfigCommPool-r14 { |  |  |  |
| SL-V2X-PreconfigCommPool-r14[1] | SL-V2X-PreconfigCommPool-r14-DEFAULT using condition BITMAP_6 and exception listed in Table 6.6.2.1G.3.4.3-2 |  |  |
| } |  |  |  |
| v2x-ResourceSelectionConfig | SL-CommTxPoolSensingConfig-r14-DEFAULT with exception listed in Table 6.6.2.1G.3.4.3-3 |  |  |
| syncPriority-r14 | GNSS |  |  |
| } |  |  |  |

Table 6.6.2.1G.3.4.3-2: SL-V2X-PreconfigCommPool-r14-DEFAULT

| Derivation Path: 36.331 clause 6.3.8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-CommResroucePoolV2X-r14-DEFAULT ::= SEQUENCE { |  |  |  |
| sl-Subframe-r14 CHOICE { |  |  |  |
| bs20-r14 | 11111111111111111111 |  | BITMAP_6 |
| } |  |  |  |
| sizeSubchannel-r14 | n1 |  | NRB_alloc = 1 |
|  | n12 |  | NRB_alloc = 12 |
|  | n48 |  | NRB_alloc = 48, NRB_alloc = 96 |
| numSubchannel-r14 | n1 |  |  |
| startRB-Subchannel-r14 | According to the test configuration in table 6.6.2.1G.3.4.1-1 |  |  |
| dataTxParameters-r14 SEQUENCE { |  |  |  |
| p0-r12 | 31 | It’s set to the maximum value to disable its effect on transmission power. |  |
| } |  |  |  |
| syncAllowed-r14 ::= SEQUENCE { |  |  |  |
| gnss-Sync-r14 | True | The resource of CC1 can be used if the UE is directly or indirectly synchronized to GNSS | CC with LCRB >0 |
| enb-Sync-r14 | True | The resource of CC1 can be used if the UE is directly or indirectly synchronized to enb | CC with LCRB =0 |
| } |  |  |  |
| } |  |  |  |

Table 6.6.2.1G.3.4.3-3: SL-PSSCH-TxParameters-r14

| Derivation Path: 36.331 clause 6.3.8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-PSSCH-TxParameters-r14 ::= SEQUENCE { |  |  |  |
| minMCS-PSSCH-r14 | 0 |  | QPSK |
|  | 11 |  | 16QAM |
| maxMCS-PSSCH-r14 | 10 |  | QPSK |
|  | 19 |  | 16QAM |
| } |  |  |  |

6.6.2.1G.3.5	Test Requirement

The power of any UE emission shall fulfil requirements in Table 6.6.2.1G.3.5-1 Bandwidth Class B.

Table 6.6.2.1G.3.5-1: General E-UTRA spectrum emission mask for CA Bandwidth Class B, E-UTRA bands > 5GHz

| Spectrum emission limit [dBm]/BWChannel_CA |  |  |
| --- | --- | --- |
| ΔfOOB (MHz) | 50RB+50RB (19.9 MHz) | Measurement bandwidth |
| 0-1 | 19 | 30 kHz |
| 1-5 | 8 | 1 MHz |
| 5-14.95 | 11 | 1 MHz |
| 14.95-19.90 | 11 | 1 MHz |
| 19.90-19.95 | 23 | 1 MHz |
| 19.95-24.90 | 23 | 1 MHz |

#### 6.6.2.1_s	Spectrum Emission Mask for subslot/slot TTI

6.6.2.1_s.1	Test purpose

To verify that the power of any UE emission shall not exceed specified lever for the specified channel bandwidth.

6.6.2.1_s.2	Test applicability

This test case applies to all types of E-UTRA Power Class 3 UE release 15 and forward that support subslot/slot TTI.

6.6.2.1_s.3	Minimum conformance requirements

The spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the edge of the assigned E-UTRA channel bandwidth. For frequencies greater than (ΔfOOB) as specified in Table 6.6.2.1_s.3-1 the spurious requirements in clause 6.6.3 are applicable.

The power of any UE emission shall not exceed the levels specified in Table 6.6.2.1_s.3-1 for the specified channel bandwidth.

Table 6.6.2.1_s.3-1: General E-UTRA spectrum emission mask

| Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -10 | -13 | -15 | -18 | -20 | -21 | 30 kHz |
| 1-2.5 | -10 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.5-2.8 | -25 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.8-5 |  | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 5-6 |  | -25 | -13 | -13 | -13 | -13 | 1 MHz |
| 6-10 |  |  | -25 | -13 | -13 | -13 | 1 MHz |
| 10-15 |  |  |  | -25 | -13 | -13 | 1 MHz |
| 15-20 |  |  |  |  | -25 | -13 | 1 MHz |
| 20-25 |  |  |  |  |  | -25 | 1 MHz |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

The normative reference for this requirement is TS 36.101 [2] clause 6.6.2.1.

6.6.2.1_s.4	Test description

6.6.2.1_s.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.1_s.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2 respectively. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.1_s.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  | NC |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  | Lowest, 5MHz, 10MHz, Highest |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
|  | Downlink Configuration | Uplink Configuration |  |  |
| Ch BW | N/A for SEM testing | Mod'n | RB allocation |  |
|  |  |  | FDD | TDD |
| 1.4MHz |  | QPSK | 6 | 6 |
| 1.4MHz |  | QPSK | 5 | 5 |
| 1.4MHz |  | 16QAM | 5 | 5 |
| 1.4MHz |  | 16QAM | 6 | 6 |
| 3MHz |  | QPSK | 15 | 15 |
| 3MHz |  | QPSK | 4 | 4 |
| 3MHz |  | 16QAM | 4 | 4 |
| 3MHz |  | 16QAM | 15 | 15 |
| 5MHz |  | QPSK | 25 | 25 |
| 5MHz |  | QPSK | 8 | 8 |
| 5MHz |  | 16QAM | 8 | 8 |
| 5MHz |  | 16QAM | 25 | 25 |
| 10MHz |  | QPSK | 50 | 50 |
| 10MHz |  | QPSK | 12 | 12 |
| 10MHz |  | 16QAM | 12 | 12 |
| 10MHz |  | 16QAM | 50 (Note 4) | 50 (Note 4) |
| 15MHz |  | QPSK | 75 | 75 |
| 15MHz |  | QPSK | 16 | 16 |
| 15MHz |  | 16QAM | 16 | 16 |
| 15MHz |  | 16QAM | 75 (Note 4) | 75 (Note 4) |
| 20MHz |  | QPSK | 100 | 100 |
| 20MHz |  | QPSK | 18 | 18 |
| 20MHz |  | 16QAM | 18 | 18 |
| 20MHz |  | 16QAM | 100 (Note 4) | 100 (Note 4) |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The allowed MPR for maximum output power UE might apply is described in clause 6.2.3_s.3. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (max+1 - RB allocation) of the channel bandwidth. Note 4: Applies only for UE-Categories ≥2 |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure A.3.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.1_s.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.1_s.4.3.

6.6.2.1_s.4.2	Test procedure

1.	SS sends uplink scheduling information via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.6.2.1_s.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Table 6.2.3_s.5-1 for Power Class 3 UEs. The period of the measurement shall be at least the continuous duration of 7 OS (0.5ms) for slot TTI and 2/3 OS for subslot TTI. For TDD slots with transient periods are not under test.

4.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to table 6.6.2.1_s.5-1 or 6.6.2.1_s.5-2 and using a rms detector, as applicable. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.6.2.1_s.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the following exceptions:

Table 6.6.2.1_s.4.3-1: PhysicalConfigDedicated-DEFAULT (preamble)

| Derivation Path: 36.508 Table 4.8.2.1.6-1 with condition Short-TTI |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |  |
| PhysicalConfigDedicatedSTTI-r15 SEQUENCE { |  |  |  |  |
| shortTTI-r15 SEQUENCE { |  |  |  |  |
| dl-STTI-Length-r15 |  | subslot |  | FDD_sTTI-22 OR FDD_sTTI-27 |
| ul-STTI-Length-r15 |  | subslot |  | FDD_sTTI-22 |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD_sTTI-22 | FDD cell environment and UEs supporting {subslot, subslot} combination |
| FDD_sTTI-27 | FDD cell environment and UEs supporting {subslot, slot} combination |

6.6.2.1_s.5	Test requirements

The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.3_s.5-1 as appropriate, and the power of any UE emission shall fulfil requirements in Table.6.6.2.1_s.5-1 or Table.6.6.2.1_s.5-2, as applicable.

Table 6.6.2.1_s.5-1: General E-UTRA spectrum emission mask, E-UTRA bands ≤ 3GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.5 | -11.5 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 1-2.5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 6-10 |  |  | -23.5 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.5 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.5 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

Table 6.6.2.1_s.5-2: General E-UTRA spectrum emission mask, 3GHz < E-UTRA bands ≤ 4.2GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.2 | -11.2 | -13.2 | -16.2 | -18.2 | -19.2 | 30 kHz |
| 1-2.5 | -8.2 | -8.2 | -8.2 | -8.2 | -8.2 | -8.2 | 1 MHz |
| 2.5-2.8 | -23.2 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.2 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 6-10 |  |  | -23.2 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.2 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.2 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.2 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

#### 6.6.2.2	Additional Spectrum Emission Mask

##### 6.6.2.2.1	Test purpose

To verify that the power of any UE emission shall not exceed specified level for the specified channel bandwidth under the deployment scenarios where additional requirements are specified.

##### 6.6.2.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 8 and forward.

##### 6.6.2.2.3	Minimum conformance requirements

6.6.2.2.3.1	Minimum requirement (network signalled value "NS_03", “NS_11”, “NS_20” and “NS_21”)

When "NS_03", “NS_11”, “NS_20” or “NS_21” is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.2.2.3.1-1.

Table 6.6.2.2.3.1-1: Additional requirements (network signalled value "NS_03", “NS_11”, “NS_20” and “NS_21”)

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -10 | -13 | -15 | -18 | -20 | -21 | 30 kHz |
| 1-2.5 | -13 | -13 | -13 | -13 | -13 | -13 | 1 MHz |
| 2.5-2.8 | -25 | -13 | -13 | -13 | -13 | -13 | 1 MHz |
| 2.8-5 |  | -13 | -13 | -13 | -13 | -13 | 1 MHz |
| 5-6 |  | -25 | -13 | -13 | -13 | -13 | 1 MHz |
| 6-10 |  |  | -25 | -13 | -13 | -13 | 1 MHz |
| 10-15 |  |  |  | -25 | -13 | -13 | 1 MHz |
| 15-20 |  |  |  |  | -25 | -13 | 1 MHz |
| 20-25 |  |  |  |  |  | -25 | 1 MHz |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

The normative reference for this requirement is TS 36.101 [2] clause 6.6.2.2.1.

6.6.2.2.3.2	Minimum requirement (network signalled value "NS_04")

When "NS_04" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.2.2.3.2-1. or Table 6.6.2.2.3.2-2

Table 6.6.2.2.3.2-1: Additional requirements (network signalled value "NS_04") (Rel-11 and earlier)

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -10 | -13 | -15 | -18 | -20 | -21 | 30 kHz |
| 1-2.5 | -13 | -13 | -13 | -13 | -13 | -13 | 1 MHz |
| 2.5-2.8 | -25 | -13 | -13 | -13 | -13 | -13 | 1 MHz |
| 2.8-5 |  | -13 | -13 | -13 | -13 | -13 | 1 MHz |
| 5-6 |  | -25 | -25 | -25 | -25 | -25 | 1 MHz |
| 6-10 |  |  | -25 | -25 | -25 | -25 | 1 MHz |
| 10-15 |  |  |  | -25 | -25 | -25 | 1 MHz |
| 15-20 |  |  |  |  | -25 | -25 | 1 MHz |
| 20-25 |  |  |  |  |  | -25 | 1 MHz |

Table 6.6.2.2.3.2-2: Additional requirements (network signalled value "NS_04") (Rel-12 and onwards)

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -15 | -18 | -20 | -21 | 30 kHz |
| 1-2.5 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.5-2.8 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.8-5 | -10 | -10 | -10 | -10 | 1 MHz |
| 5-6 | -13 | -13 | -13 | -13 | 1 MHz |
| 6-9 | -25 | -13 | -13 | -13 | 1 MHz |
| 9-10 | -25 | -25 | -13 | -13 | 1 MHz |
| 10-13.5 |  | -25 | -13 | -13 | 1 MHz |
| 13.5-15 |  | -25 | -25 | -13 | 1 MHz |
| 15-18 |  |  | -25 | -13 | 1 MHz |
| 18-20 |  |  | -25 | -25 | 1 MHz |
| 20-25 |  |  |  | -25 | 1 MHz |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

The normative reference for this requirement is TS 36.101 [2] clause 6.6.2.2.2.

6.6.2.2.3.3	Minimum requirement (network signalled value "NS_06" or NS_07)

When "NS_06" or "NS_07" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.2.2.3.3-1.

Table 6.6.2.2.3.3-1: Additional requirements (network signalled value "NS_06" or "NS_07")

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | Measurement bandwidth |
| 0-0.1 | -13 | -13 | -15 | -18 | 30 kHz |
| 0.1-1 | -13 | -13 | -13 | -13 | 100 kHz |
| 1-2.5 | -13 | -13 | -13 | -13 | 1 MHz |
| 2.5-2.8 | -25 | -13 | -13 | -13 | 1 MHz |
| 2.8-5 |  | -13 | -13 | -13 | 1 MHz |
| 5-6 |  | -25 | -13 | -13 | 1 MHz |
| 6-10 |  |  | -25 | -13 | 1 MHz |
| 10-15 |  |  |  | -25 | 1 MHz |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

The normative reference for this requirement is TS 36.101 [2] clause 6.6.2.2.3.

6.6.2.2.3.4	Minimum requirement (network signalled value “NS_27”)

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When “NS_27” is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.2.2.3.4-1.

Table 6.6.2.2.3.4-1: Additional requirements (network signalled value "NS_27")

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -13 | -13 | -13 | -13 | Note 1 |
| 1-10 | -13 | -13 | -13 | -13 | 1 MHz |
| 10-15 |  | -25 | -25 | -25 | 1 MHz |
| 15-20 |  |  | -25 | -25 | 1 MHz |
| 20-25 |  |  |  | -25 | 1 MHz |
| Note 1: The measurement bandwidth is 1% of the applicable E-UTRA channel bandwidth. |  |  |  |  |  |

The normative reference for this requirement is TS 36.101 [2] clause 6.6.2.2.5.

6.6.2.2.3.5	Minimum requirement (network signalled value “NS_35”)

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When "NS_35" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.2.2.3.5-1.

Table 6.6.2.2.3.5-1: Additional requirements (network signalled value "NS_35")

| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth (unless otherwise stated) |
| --- | --- | --- | --- | --- | --- |
| 0-0.1 | -15 | -18 | -20 | -21 | 30 kHz |
| 0.1-6 | -13 | -13 | -13 | -13 | 100 kHz |
| 6-10 | -251 | -13 | -13 | -13 | 100 kHz |
| 10-15 |  | -251 | -13 | -13 | 100 kHz |
| 15-20 |  |  | -251 | -13 | 100 kHz |
| 20-25 |  |  |  | -25 | 1 MHz |
| Note 1: The measurement bandwidth shall be 1 MHz |  |  |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

The normative reference for this requirement is TS 36.101 [2] clause 6.6.2.2.7.

6.6.2.2.3.6	Void

##### 6.6.2.2.4	Test description

6.6.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Tables 6.6.2.2.4.1-1 through 6.6.2.2.4.1-11. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2 respectively. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.2.4.1-1: Test Configuration Table (network signalled value "NS_03")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Lowest, 5MHz, 10MHz, Highest |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |  |
| Ch BW | Mod'n | RB allocation |  | Mod'n | RB allocation |  |
|  |  | FDD | TDD |  | FDD | TDD |
| 1.4MHz | N/A for Additional Spectrum Emission Mask testing. |  |  | QPSK | 6 | 6 |
| 1.4MHz |  |  |  | QPSK | 5 | 5 |
| 1.4MHz |  |  |  | 16QAM (Note 4) | 5 | 5 |
| 3MHz |  |  |  | QPSK | 15 | 15 |
| 3MHz |  |  |  | QPSK | 4 | 4 |
| 3MHz |  |  |  | 16QAM (Note 4) | 15 | 15 |
| 3MHz |  |  |  | 16QAM (Note 4) | 4 | 4 |
| 5MHz |  |  |  | QPSK | 25 | 25 |
| 5MHz |  |  |  | QPSK | 8 | 8 |
| 5MHz |  |  |  | QPSK | 6 | 6 |
| 5MHz |  |  |  | 16QAM (Note 4) | 25 | 25 |
| 5MHz |  |  |  | 16QAM (Note 4) | 8 | 8 |
| 10MHz |  |  |  | QPSK | 50 | 50 |
| 10MHz |  |  |  | QPSK | 12 | 12 |
| 10MHz |  |  |  | QPSK | 6 | 6 |
| 10MHz |  |  |  | 16QAM (Note 4) | 50 (Note 3) | 50 (Note 3) |
| 10MHz |  |  |  | 16QAM (Note 4) | 12 | 12 |
| 15MHz |  |  |  | QPSK | 75 | 75 |
| 15MHz |  |  |  | QPSK | 16 | 16 |
| 15MHz |  |  |  | QPSK | 8 | 8 |
| 15MHz |  |  |  | 16QAM (Note 4) | 75 (Note 3) | 75 (Note 3) |
| 15MHz |  |  |  | 16QAM (Note 4) | 16 | 16 |
| 20MHz |  |  |  | QPSK | 100 | 100 |
| 20MHz |  |  |  | QPSK | 18 | 18 |
| 20MHz |  |  |  | QPSK | 10 | 10 |
| 20MHz |  |  |  | 16QAM (Note 4) | 100 (Note 3) | 100 (Note 3) |
| 20MHz |  |  |  | 16QAM (Note 4) | 18 | 18 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 3: Applies only for UE-Categories ≥2. Note 4: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |  |

Table 6.6.2.2.4.1-2: Test Configuration Table (network signalled value "NS_04", Power Class 2)

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | Normal |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Low Range, Mid Range, High Range Additional test frequencies: For 5 MHz bandwidth: 2500.6 MHz, NUL= 39696 For 10 MHz bandwidth: 2504.1 MHz, NUL= 39731 For 15 MHz bandwidth: 2510.9 MHz, NUL= 39799 For 20 MHz bandwidth: 2517.6 MHz, NUL= 39866 |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 5MHz, 10MHz, 15MHz, 20MHz |  |  |
| Test Parameters for NS_04 A-MPR |  |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID | Ch BW | Mod'n | RB allocation TDD | Mod'n | RB allocation TDD | RBstart TDD |
| 1 (Note 1) | 5MHz | N/A for A-MPR testing |  | QPSK | 1 | 0 |
| 2 (Note 1) | 5MHz |  |  | QPSK | 16 | 9 |
| 3 (Note 2) | 5MHz |  |  | QPSK | 25 | 0 |
| 4 (Note 1) | 5MHz |  |  | 16QAM | 16 | 9 |
| 5 (Note 2) | 5MHz |  |  | 16QAM | 25 | 0 |
| 6 (Note 1) | 10MHz |  |  | QPSK | 1 | 0 |
| 7 (Note 1) | 10MHz |  |  | QPSK | 1 | 49 |
| 8 (Note 1) | 10MHz |  |  | QPSK | 24 | 0 |
| 9 (Note 1) | 10MHz |  |  | QPSK | 40 | 10 |
| 10 | 10MHz |  |  | QPSK | 50 | 0 |
| 11 | 10MHz |  |  | 16QAM | 50 | 0 |
| 12 (Note 1) | 15MHz |  |  | QPSK | 1 | 0 |
| 13 (Note 1) | 15MHz |  |  | QPSK | 1 | 74 |
| 14 (Note 1) | 15MHz |  |  | QPSK | 32 | 0 |
| 15 (Note 1) | 15MHz |  |  | QPSK | 60 | 15 |
| 16 | 15MHz |  |  | QPSK | 75 | 0 |
| 17 | 15MHz |  |  | 16QAM | 75 | 0 |
| 18 (Note 1) | 20MHz |  |  | QPSK | 1 | 0 |
| 19 (Note 1) | 20MHz |  |  | QPSK | 1 | 99 |
| 20 (Note 1) | 20MHz |  |  | QPSK | 36 | 0 |
| 21 (Note 1) | 20MHz |  |  | QPSK | 75 | 25 |
| 22 | 20MHz |  |  | QPSK | 100 | 0 |
| 23 | 20MHz |  |  | 16QAM | 100 | 0 |
| Note 1: Only applicable for Low Range. Note 2: Only applicable for Mid, High Range and additional test frequencies. |  |  |  |  |  |  |

Table 6.6.2.2.4.1-3: Test Configuration Table (network signalled value "NS_04", Power Class 3)

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range Additional test frequencies for Rel-12 to Rel-13 and bit 2 of modifiedMPRbehavior = 0: For 5 MHz bandwidth: 2499.6 MHz, NUL= 39686 For 10 MHz bandwidth: 2504.1 MHz, NUL= 39731 For 15 MHz bandwidth: 2510.9 MHz, NUL= 39799 For 20 MHz bandwidth: 2517.6 MHz, NUL= 39866 Additional test frequencies for Rel-12 and later and bit 2 of modifiedMPRbehavior = 1: For 5 MHz bandwidth: 2500.6 MHz, NUL= 39696 For 10 MHz bandwidth: 2504.1 MHz, NUL= 39731 For 15 MHz bandwidth: 2510.9 MHz, NUL= 39799 For 20 MHz bandwidth: 2517.6 MHz, NUL= 39866 |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 5MHz, 10 MHz, 15 MHz, 20MHz |  |  |
| Test Parameters for NS_04 A-MPR |  |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configura-tion ID | Ch BW | Mod'n | RB allocation TDD | Mod'n | RB allocation TDD | RBstart TDD |
| 1 (Note 6) | 5MHz | N/A for A-MPR testing |  | QPSK | 25 | 0 |
| 2 (Note 6) | 5MHz |  |  | QPSK | 8 | Note 3 |
| 3(Note 6) | 5MHz |  |  | QPSK | 6 | Note 3 |
| 4 (Note 6) | 5MHz |  |  | 16QAM (Note 5) | 25 | 0 |
| 5 (Note 6) | 5MHz |  |  | 16QAM (Note 5) | 8 | Note 3 |
| 6 (Note 6) | 10MHz |  |  | QPSK | 1 | 0 |
| 7 (Note 6) | 10MHz |  |  | QPSK | 12 | 0 |
| 8 (Note 6) | 10MHz |  |  | QPSK | 50 | 0 |
| 9 Note 6) | 10MHz |  |  | 16QAM (Note 5) | 50 (Note 4) | 0 |
| 10 (Note 6) | 10MHz |  |  | QPSK | 24 | 13 |
| 11 (Note 6) | 10MHz |  |  | 16QAM (Note 5) | 24 | 13 |
| 12 (Note 6) | 10MHz |  |  | QPSK | 36 | 13 |
| 13 (Note 6) | 10MHz |  |  | QPSK | 12 | 37 |
| 14 (Note 6) | 10MHz |  |  | QPSK | 1 | 49 |
| 15 (Note 6) | 15MHz |  |  | QPSK | 1 | 0 |
| 16 (Note 6) | 15MHz |  |  | QPSK | 16 | 0 |
| 17 (Note 6) | 15MHz |  |  | QPSK | 75 | 0 |
| 18 (Note 6) | 15MHz |  |  | 16QAM (Note 5) | 75 (Note 4) | 0 |
| 19 (Note 6) | 15MHz |  |  | QPSK | 36 | 19 |
| 20 (Note 6) | 15MHz |  |  | 16QAM (Note 5) | 36 (Note 4) | 19 |
| 21 (Note 6) | 15MHz |  |  | QPSK | 50 | 19 |
| 22 (Note 6) | 15MHz |  |  | QPSK | 18 | 56 |
| 23 (Note 6) | 15MHz |  |  | QPSK | 1 | 74 |
| 24 (Note 6) | 20MHz |  |  | QPSK | 1 | 0 |
| 25 (Note 6) | 20MHz |  |  | QPSK | 18 | 0 |
| 26 (Note 6) | 20MHz |  |  | QPSK | 100 | 0 |
| 27 (Note 6) | 20MHz |  |  | 16QAM (Note 5) | 100 (Note 4) | 0 |
| 28 (Note 6) | 20MHz |  |  | QPSK | 50 | 25 |
| 29 (Note 6) | 20MHz |  |  | 16QAM (Note 5) | 50 (Note 4) | 25 |
| 30 (Note 6) | 20MHz |  |  | QPSK | 75 | 25 |
| 31 (Note 6) | 20MHz |  |  | QPSK | 25 | 75 |
| 32 (Note 6) | 20MHz |  |  | QPSK | 1 | 99 |
| 33 (Note 8) | 5MHz |  |  | QPSK | 1 | 0 |
| 34 (Note 8) | 5MHz |  |  | QPSK | 16 | 9 |
| 35 (Note 9) | 5MHz |  |  | QPSK | 25 | 0 |
| 36 (Note 8) | 5MHz |  |  | 16QAM (Note 5) | 16 | 9 |
| 37 (Note 9) | 5MHz |  |  | 16QAM (Note 5) | 25 | 0 |
| 38 (Note 8) | 10MHz |  |  | QPSK | 1 | 0 |
| 39 (Note 8) | 10MHz |  |  | QPSK | 1 | 49 |
| 40 (Note 8) | 10MHz |  |  | QPSK | 24 | 0 |
| 41 (Note 8) | 10MHz |  |  | QPSK | 40 | 10 |
| 42 (Note 7) | 10MHz |  |  | QPSK | 50 | 0 |
| 43 (Note 7) | 10MHz |  |  | 16QAM (Note 5) | 50 | 0 |
| 44 (Note 8) | 15MHz |  |  | QPSK | 1 | 0 |
| 45 (Note 8) | 15MHz |  |  | QPSK | 1 | 74 |
| 46 (Note 8) | 15MHz |  |  | QPSK | 32 | 0 |
| 47 (Note 8) | 15MHz |  |  | QPSK | 60 | 15 |
| 48 (Note 7) | 15MHz |  |  | QPSK | 75 | 0 |
| 49 (Note 7) | 15MHz |  |  | 16QAM (Note 5) | 75 | 0 |
| 50 (Note 8) | 20MHz |  |  | QPSK | 1 | 0 |
| 51 (Note 8) | 20MHz |  |  | QPSK | 1 | 99 |
| 52 (Note 8) | 20MHz |  |  | QPSK | 36 | 0 |
| 53 (Note 8) | 20MHz |  |  | QPSK | 75 | 25 |
| 54 (Note 7) | 20MHz |  |  | QPSK | 100 | 0 |
| 55 (Note 7) | 20MHz |  |  | 16QAM (Note 5) | 100 | 0 |
| Note 1: Test Channel Bandwidths are checked separately for E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The configuration ID will be used to map the applicable Test Configuration to be corresponding Test Requirement in subclause 6.2.4 as not all combinations are necessarily required based on the applicability of the UE. Note 3: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 4: Applies only for UE-Categories ≥2. Note 5: The 16QAM configurations shall not apply to UEs which support UL 64QAM. Note 6: Applies only for Rel-11 and earlier. Note 7: Applies only for Rel-12 and later. Note 8: Applies only for Rel-12 and later and Low Range. Note 9: Applies only for Rel-12 and later and Mid, High Range and additional test frequencies. |  |  |  |  |  |  |

Table 6.6.2.2.4.1-4: Test Configuration Table (network signalled value "NS_06", Power Class 1)

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | Normal |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Lowest, 5MHz, 10MHz, Highest |  |
| Test Parameters for NS_06 A-MPR |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |
| Configuration ID | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD |
| 1 | 5MHz | N/A for A-MPR testing |  | QPSK | 25 |
| 2 | 5MHz |  |  | QPSK | 8 |
| 3 | 5MHz |  |  | 16QAM | 8 |
| 4 | 10MHz |  |  | QPSK | 50 |
| 5 | 10MHz |  |  | QPSK | 12 |
| 6 | 10MHz |  |  | 16QAM | 12 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4_1.5 as not all combinations are necessarily required based on the applicability of the UE. Note 3: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. |  |  |  |  |  |

Table 6.6.2.2.4.1-5: Test Configuration Table (network signalled value "NS_06", Power Class 3)

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Lowest, 5MHz, 10MHz |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |  |
| Ch BW | Mod'n | RB allocation |  | Mod'n | RB allocation |  |
|  |  | FDD | TDD |  | FDD | TDD |
| 1.4MHz | N/A for Additional Spectrum Emission Mask testing. |  |  | QPSK | 6 | NA |
| 1.4MHz |  |  |  | QPSK | 5 |  |
| 1.4MHz |  |  |  | 16QAM (Note 3) | 5 |  |
| 3MHz |  |  |  | QPSK | 15 |  |
| 3MHz |  |  |  | QPSK | 4 |  |
| 3MHz |  |  |  | 16QAM (Note 3) | 4 |  |
| 5MHz |  |  |  | QPSK | 25 |  |
| 5MHz |  |  |  | QPSK | 8 |  |
| 5MHz |  |  |  | 16QAM (Note 3) | 8 |  |
| 10MHz |  |  |  | QPSK | 50 |  |
| 10MHz |  |  |  | QPSK | 12 |  |
| 10MHz |  |  |  | 16QAM (Note 3) | 12 |  |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 3: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |  |

Table 6.6.2.2.4.1-6: Test Configuration Table (network signalled value "NS_07")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  |  |  | Mid range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 10MHz |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Test Number | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD | RBstart |
| 1 | 10MHz | N/A for Additional Spectrum Emission Mask testing. |  | QPSK | 1 | 0 |
| 2 | 10MHz |  |  | QPSK | 8 | 0 |
| 3 | 10MHz |  |  | QPSK | 6 | 13 |
| 4 | 10MHz |  |  | QPSK | 20 | 13 |
| 5 | 10MHz |  |  | QPSK | 12 | 13 |
| 6 | 10MHz |  |  | 16QAM (Note 2) | 36 (Note 1) | 13 |
| 7 | 10MHz |  |  | QPSK | 16 | 19 |
| 8 | 10MHz |  |  | QPSK | 12 | 19 |
| 9 | 10MHz |  |  | 16QAM (Note 2) | 16 | 19 |
| 10 | 10MHz |  |  | QPSK | 30 | 19 |
| 11 | 10MHz |  |  | 16QAM (Note 2) | 30 (Note 1) | 19 |
| 12 | 10MHz |  |  | QPSK | 6 | 43 |
| 13 | 10MHz |  |  | QPSK | 2 | 48 |
| 14 | 10MHz |  |  | QPSK | 50 | 0 |
| 15 | 10MHz |  |  | QPSK | 12 | 0 |
| 16 | 10MHz |  |  | 16QAM (Note 2) | 50 (Note 1) | 0 |
| Note 1: Applies only for UE-Categories ≥2. Note 2: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |  |

Table 6.6.2.2.4.1-7: Test Configuration Table (network signalled value "NS_11")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | Normal |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range For 3 MHz Channel Bandwidth: a. 2001.5 MHz (NUL = 25515) b. 2004.5 MHz (NUL = 25545) For 5 MHz Channel Bandwidth a. 2002.5 MHz (NUL = 25525) b. 2004.5 MHz (NUL = 25545) c. 2007.5 MHz (NUL = 25575) For 10 MHz Channel Bandwidth a. 2005 MHz (NUL = 25550) b. 2005.5 MHz (NUL = 25555) c. 2015 MHz (NUL = 25650) For 15 MHz Channel Bandwidth a. 2007.5 MHz (NUL = 25575) b. 2012.5 MHz (NUL = 25625) For 20 MHz Channel Bandwidth a. 2010 MHz (NUL = 25600) |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 3MHz, 5MHz, 10MHz, 15MHz, 20MHz |  |
| Test Parameters for NS_11 A-MPR |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |
| Configuration ID | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD |
| 1 | 3MHz | N/A for A-MPR testing |  | QPSK | 6 |
| 2 | 3MHz |  |  | QPSK | 15 |
| 3 | 3MHz |  |  | 16QAM (Note 4) | 6 |
| 4 | 3MHz |  |  | 16QAM (Note 4) | 15 |
| 5 | 5MHz |  |  | QPSK | 1 |
| 6 | 5MHz |  |  | QPSK | 8 |
| 7 | 5MHz |  |  | QPSK | 25 |
| 8 | 5MHz |  |  | 16QAM (Note 4) | 8 |
| 9 | 5MHz |  |  | 16QAM (Note 4) | 25 |
| 10 | 10MHz |  |  | QPSK | 1 |
| 11 | 10MHz |  |  | QPSK | 12 |
| 12 | 10MHz |  |  | QPSK | 50 |
| 13 | 10MHz |  |  | 16QAM (Note 4) | 12 |
| 14 | 10MHz |  |  | 16QAM (Note 4) | 50 |
|  |  |  |  |  | (Note 3) |
| 15 | 15MHz |  |  | QPSK | 1 |
| 16 | 15MHz |  |  | QPSK | 8 |
| 17 | 15MHz |  |  | QPSK | 25 |
| 18 | 15MHz |  |  | QPSK | 30 |
| 19 | 15MHz |  |  | QPSK | 75 |
| 20 | 15MHz |  |  | 16QAM (Note 4) | 8 |
| 21 | 15MHz |  |  | 16QAM (Note 4) | 25 |
| 22 | 15MHz |  |  | 16QAM (Note 4) | 30 |
| 23 | 15MHz |  |  | 16QAM (Note 4) | 75 |
| 24 | 20MHz |  |  | QPSK | 1 |
| 25 | 20MHz |  |  | QPSK | 10 |
| 26 | 20MHz |  |  | QPSK | 25 |
| 27 | 20MHz |  |  | QPSK | 100 |
| 28 | 20MHz |  |  | 16QAM (Note 4) | 10 |
| 29 | 20MHz |  |  | 16QAM (Note 4) | 25 |
| 30 | 20MHz |  |  | 16QAM (Note 4) | 100 |
| Note 1: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4.5 as not all combinations are necessarily required based on the applicability of the UE. Note 2: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 3: Applies only for UE-Categories ≥2. Note 4: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |

Table 6.6.2.2.4.1-8: Test Configuration Table (network signalled value "NS_20")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | Normal |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range For 5 MHz Channel Bandwidth a. 2002.5 MHz (NUL = 25525) b. 2007.5 MHz (NUL = 25575) c. 2012.5 MHz (NUL = 25625) d. 2017.5 MHz (NUL = 25675) For 10 MHz Channel Bandwidth a. 2005 MHz (NUL = 25550) b. 2015 MHz (NUL = 25650) For 15 MHz Channel Bandwidth a. 2012.5 MHz (NUL = 25625) For 20 MHz Channel Bandwidth a. 2010 MHz (NUL = 25600) |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 5MHz, 10MHz, 15MHz, 20MHz |  |
| Test Parameters for NS_20 A-MPR |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |
| Configuration ID | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD |
| 1 | 5MHz | N/A for A-MPR testing |  | QPSK | 8 |
| 2 | 5MHz |  |  | QPSK | 15 |
| 3 | 5MHz |  |  | QPSK | 25 |
| 4 | 5MHz |  |  | 16QAM (Note 4) | 15 |
| 5 | 5MHz |  |  | 16QAM (Note 4) | 25 |
| 6 | 10MHz |  |  | QPSK | 8 |
| 7 | 10MHz |  |  | QPSK | 12 |
| 8 | 10MHz |  |  | QPSK | 50 |
| 9 | 10MHz |  |  | 16QAM (Note 4) | 12 |
| 10 | 10MHz |  |  | 16QAM (Note 4) | 50 |
|  |  |  |  |  | (Note 3) |
| 11 | 15MHz |  |  | QPSK | 6 |
| 12 | 15MHz |  |  | QPSK | 25 |
| 13 | 15MHz |  |  | QPSK | 36 |
| 14 | 15MHz |  |  | QPSK | 75 |
| 15 | 15MHz |  |  | 16QAM (Note 4) | 25 |
| 16 | 15MHz |  |  | 16QAM (Note 4) | 36 |
| 17 | 15MHz |  |  | 16QAM (Note 4) | 75 |
| 18 | 20MHz |  |  | QPSK | 8 |
| 19 | 20MHz |  |  | QPSK | 18 |
| 20 | 20MHz |  |  | QPSK | 25 |
| 21 | 20MHz |  |  | QPSK | 75 |
| 22 | 20MHz |  |  | QPSK | 100 |
| 23 | 20MHz |  |  | 16QAM (Note 4) | 18 |
| 24 | 20MHz |  |  | 16QAM (Note 4) | 25 |
| 25 | 20MHz |  |  | 16QAM (Note 4) | 75 |
| 26 | 20MHz |  |  | 16QAM (Note 4) | 100 |
| Note 1: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4.5 as not all combinations are necessarily required based on the applicability of the UE. Note 2: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 3: Applies only for UE-Categories ≥2. Note 4: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |

Table 6.6.2.2.4.1-9: Test Configuration Table (network signalled value "NS_21")

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  |  |  | NC |  |  |  |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  |  |  |  |  |  | Low range or High range |  |  |  |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  |  |  | 5 MHz, 10 MHz |  |  |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Downlink Configuration |  |  | Uplink Configuration |  |  |  |  |  |
| Configuration ID |  | Ch BW |  | Mod'n |  | RB allocation | Mod'n |  | RB allocation FDD |  | RBstart FDD |  |
| 1 (note 2) |  | 5 MHz |  |  |  |  | QPSK |  | 1 |  | 0 |  |
| 2 (note 2) |  | 5 MHz |  |  |  |  | QPSK |  | 25 |  | 0 |  |
| 3 (note 2) |  | 10 MHz |  |  |  |  | QPSK |  | 1 |  | 0 |  |
| 4 (note 3) |  | 10 MHz |  |  |  |  | QPSK |  | 1 |  | 49 |  |
| 5 (note 2) |  | 10 MHz |  |  |  |  | QPSK |  | 15 |  | 0 |  |
| 6 (note 3) |  | 10 MHz |  |  |  |  | QPSK |  | 3 |  | 47 |  |
| 7 (note 3) |  | 10 MHz |  |  |  |  | QPSK |  | 50 |  | 0 |  |
| 8 (note 1, 3) |  | 10 MHz |  |  |  |  | 16QAM (Note 4) |  | 50 |  | 0 |  |
| Note 1: Applies only for UE-Categories ≥2. Note 2: Applicable only to low range frequency testing. Note 3: Applicable only to high range frequency testing. Note 4: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.6.2.2.4.1-10: Test Configuration Table (network signalled value "NS_27")

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  | Normal |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | Refer to Frequency column in test parameters |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | 5MHz, 10MHz, 15MHz, 20MHz |  |  |
| Test Parameters for NS_27 A-MPR |  |  |  |  |  |  |  |
|  |  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID | Frequency | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD | RBstart FDD |
| 1 | Low/Mid | 5MHz | N/A for A-MPR testing |  | QPSK | 1 | 0 |
| 2 | High | 5MHz |  |  | QPSK | 1 | 24 |
| 3 | Low/Mid/High | 5MHz |  |  | QPSK | 25 | 0 |
| 4 | Low/Mid/High | 5MHz |  |  | 16QAM | 25 | 0 |
| 5 | Low/Mid | 10MHz |  |  | QPSK | 1 | 0 |
| 6 | High | 10MHz |  |  | QPSK | 1 | 49 |
| 7 | Low/Mid/High | 10MHz |  |  | QPSK | 50 | 0 |
| 8 (Note 2) | Low/Mid/High | 10MHz |  |  | 16QAM | 50 | 0 |
| 9 | Low/Mid | 15MHz |  |  | QPSK | 1 | 0 |
| 10 | Low | 15MHz |  |  | QPSK | 1 | 7 |
| 11 | Low/Mid | 15MHz |  |  | QPSK | 16 | 0 |
| 12 | High | 15MHz |  |  | QPSK | 1 | 74 |
| 13 | High | 15MHz |  |  | QPSK | 1 | 67 |
| 14 | High | 15MHz |  |  | QPSK | 16 | 74 |
| 15 | Low/Mid/High | 15MHz |  |  | QPSK | 75 | 0 |
| 16 (Note 2) | Low/Mid/High | 15MHz |  |  | 16QAM | 75 | 0 |
| 17 | Low/Mid | 20MHz |  |  | QPSK | 1 | 0 |
| 18 | Low | 20MHz |  |  | QPSK | 1 | 13 |
| 19 | Low | 20MHz |  |  | QPSK | 1 | 16 |
| 20 | Low/Mid | 20MHz |  |  | QPSK | 21 | 0 |
| 21 | High | 20MHz |  |  | QPSK | 1 | 99 |
| 22 | High | 20MHz |  |  | QPSK | 1 | 86 |
| 23 | High | 20MHz |  |  | QPSK | 1 | 83 |
| 24 | High | 20MHz |  |  | QPSK | 21 | 0 |
| 25 | Low/Mid/High | 20MHz |  |  | QPSK | 100 | 0 |
| 26 (Note 2) | Low/Mid/High | 20MHz |  |  | 16QAM | 100 | 0 |
| Note 1: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4.5 as not all combinations are necessarily required based on the applicability of the UE. Note 2: Applies only for UE-Categories ≥2. Note 3: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |  |  |

Table 6.6.2.2.4.1-11: Test Configuration Table (network signalled value "NS_35")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 5MHz, 10MHz, 15MHz, 20MHz |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |  |
| Ch BW | Mod'n | RB allocation |  | Mod'n | RB allocation |  |
|  |  | FDD | TDD |  | FDD | TDD |
| 5MHz | N/A for Additional Spectrum Emission Mask testing. |  |  | QPSK | 25 | NA |
| 5MHz |  |  |  | QPSK | 8 |  |
| 5MHz |  |  |  | 16QAM (Note 3) | 8 |  |
| 10MHz |  |  |  | QPSK | 50 |  |
| 10MHz |  |  |  | QPSK | 12 |  |
| 10MHz |  |  |  | 16QAM (Note 3) | 12 |  |
| 15MHz |  |  |  | QPSK | 75 |  |
| 15MHz |  |  |  | QPSK | 16 |  |
| 15MHz |  |  |  | 16QAM (Note 3) | 16 |  |
| 20MHz |  |  |  | QPSK | 100 |  |
| 20MHz |  |  |  | QPSK | 18 |  |
| 20MHz |  |  |  | 16QAM (Note 3) | 18 |  |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 3: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.3.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The DL Reference Measurement channels are set according to Tables 6.6.2.2.4.1-1 to 6.6.2.2.4.1-11.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.2.4.3.

6.6.2.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to applicable table from Table 6.6.2.2.4.1-1 to 6.6.2.2.4.1-11. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Table 6.2.4.5-1 to 6.2.4.5-28 or 6.2.4_1.5-1 as appropriate. The period of the measurement shall be at least one sub-frame (1ms).

4.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to Table 6.6.2.2.5.1-1, 6.6.2.2.5.2-1, 6.6.2.2.5.3-1 or Table 6.6.2.2.5.1-2, 6.6.2.2.5.2-2, 6.6.2.2.5.3-2, or 6.6.2.2.5.4-1, 6.6.2.2.5.5-1 as applicable and using a rms detector. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.6.2.2.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6, with the following exceptions for each network signalled value.

6.6.2.2.4.3.1	Message contents exceptions (network signalled value "NS_03")

1. Information element additionalSpectrumEmission is set to NS_03. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.6.2.2.4.3.1-1: SystemInformationBlockType2: Additional spurious emissions requirement

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 3 (NS_03) |  |  |

6.6.2.2.4.3.2	Message contents exceptions (network signalled value "NS_04")

1. Information element additionalSpectrumEmission is set to NS_04. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.6.2.2.4.3.2-1: SystemInformationBlockType2: Additional spurious emissions requirement

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 4 (NS_04) |  |  |

6.6.2.2.4.3.3	Message contents exceptions (network signalled value "NS_06")

1. Information element additionalSpectrumEmission is set to NS_06. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.6.2.2.4.3.3-1: SystemInformationBlockType2: Additional spurious emissions requirement

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 6 (NS_06) |  |  |

6.6.2.2.4.3.4	Message contents exceptions (network signalled value "NS_07")

1. Information element additionalSpectrumEmission is set to NS_07. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.6.2.2.4.3.4-1: SystemInformationBlockType2: Additional spurious emissions requirement

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 7 (NS_07) |  |  |

6.6.2.2.4.3.5	Message contents exceptions (network signalled value "NS_11")

1. Information element additionalSpectrumEmission is set to NS_11. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.6.2.2.4.3.5-1: SystemInformationBlockType2: Additional spurious emissions requirement

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 11 (NS_11) |  |  |

6.6.2.2.4.3.6	Message contents exceptions (network signalled value "NS_20")

1. Information element additionalSpectrumEmission is set to NS_20. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.6.2.2.4.3.6-1: SystemInformationBlockType2: Additional spurious emissions requirement

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 20 (NS_20) |  |  |

6.6.2.2.4.3.7	Message contents exceptions (network signalled value "NS_21")

1. Information element additionalSpectrumEmission is set to NS_21. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.6.2.2.4.3.7-1: SystemInformationBlockType2: Additional spurious emissions requirement

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 21 (NS_21) |  |  |

6.6.2.2.4.3.8	Message contents exceptions (network signalled value "NS_27")

1. Information element additionalSpectrumEmission is set to NS_27. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.6.2.2.4.3.8-1: SystemInformationBlockType2: Additional spurious emissions requirement

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 27 (NS_27) |  |  |

6.6.2.2.4.3.9	Message contents exceptions (network signalled value "NS_35")

1. Information element additionalSpectrumEmission is set to NS_35. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.6.2.2.4.3.8-1: SystemInformationBlockType2: Additional spurious emissions requirement

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 32 |  |  |
| additionalSpectrumEmission-v10l0 | 35 (NS_35) |  |  |

##### 6.6.2.2.5	Test requirements

6.6.2.2.5.1	Test requirements (network signalled value "NS_03", “NS_11”, “NS_20” and “NS_21”)

When "NS_03" or “NS_11” or “NS_20” or “NS_21” is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.4.5-1, 6.2.4.5-2, 6.2.4.5-11, 6.2.4.5-20 and 6.2.4.5-21 as appropriate,

and

-	the power of any UE emission shall fulfil requirements in Table 6.6.2.2.5.1-1 or 6.6.2.2.5.1-2, as applicable.

Table 6.6.2.2.5.1-1: Additional requirements (network signalled value "NS_03", “NS_11”, “NS_20” and “NS_21”), E-UTRA bands ≤ 3GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.5 | -11.5 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 1-2.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 |  |  |  |  | 1 MHz |
| 6-10 |  |  | -23.5 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.5 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.5 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_03 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

Table 6.6.2.2.5.1-2: Additional requirements (network signalled value "NS_03", “NS_11”, “NS_20” and “NS_21”), 3GHz < E-UTRA bands ≤ 4.2GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.2 | -11.2 | -13.2 | -16.2 | -18.2 | -19.2 | 30 kHz |
| 1-2.5 | -11.2 | -11.2 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 2.5-2.8 | -23.2 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.2 |  |  |  |  | 1 MHz |
| 6-10 |  |  | -23.2 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.2 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.2 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.2 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_03 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

NOTE:	(only for emission measurement) As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

6.6.2.2.5.2	Test requirements (network signalled value "NS_04")

When "NS_04" is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.4.5-3 for a Power Class 3 UE or 6.2.4_1.5-2 for a Power Class 2 UE as appropriate,

and

-	the power of any UE emission shall fulfil requirements in Table 6.6.2.2.5.2-1 or Table.6.6.2.2.5.2-3, as applicable.

Table 6.6.2.2.5.2-1: Additional requirements (network signalled value "NS_04") (Rel-11 and earlier)

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.5 | -11.5 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 1-2.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 | -23.5 | -23.5 | -23.5 | -23.5 | 1 MHz |
| 6-10 |  |  |  |  |  |  | 1 MHz |
| 10-15 |  |  |  |  |  |  | 1 MHz |
| 15-20 |  |  |  |  |  |  | 1 MHz |
| 20-25 |  |  |  |  |  |  | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_04 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

Table 6.6.2.2.5.2-2: Void

Table 6.6.2.2.3.2-3: Additional requirements (network signalled value "NS_04") (Rel-12 and onwards)

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 1-2.5 | -8.5 | --8.5 | -8.5 | -8.5 | 1 MHz |
| 2.5-2.8 |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  | 1 MHz |
| 5-6 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 6-9 | -23.5 |  |  |  | 1 MHz |
| 9-10 |  | -23.5 |  |  | 1 MHz |
| 10-13.5 |  |  |  |  | 1 MHz |
| 13.5-15 |  |  | -23.5 |  | 1 MHz |
| 15-18 |  |  |  |  | 1 MHz |
| 18-20 |  |  |  | -23.5 | 1 MHz |
| 20-25 |  |  |  |  | 1 MHz |

NOTE:	(only for emission measurement) As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

6.6.2.2.5.3	Test requirements (network signalled value "NS_06" or "NS_07")

When "NS_06" or "NS_07" is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.4.5-5, 6.2.4.5-6 and 6.2.4.5-7 as appropriate for a Power Class 3 UE or in Table 6.2.4_1.5-1 for a Power Class 1 UE,

and

-	the power of any UE emission shall fulfil requirements in Table 6.6.2.2.5.3-1 or Table.6.6.2.2.5.3-2, as applicable.

Table 6.6.2.2.5.3-1: Additional requirements (network signalled value "NS_06" or "NS_07"), E-UTRA bands ≤ 3GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | Measurement bandwidth |
| 0-0.1 | -11.5 | -11.5 | -13.5 | -16.5 | 30 kHz |
| 0.1-1 | -11.5 | -11.5 | -11.5 | -11.5 | 100 kHz |
| 1-2.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 |  |  | 1 MHz |
| 6-10 |  |  | -23.5 |  | 1 MHz |
| 10-15 |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.085 MHz. The first and last measurement position with a 100 kHz filter is at ΔfOOB equals to 0.15 MHz and 0.95 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_06 and NS_07 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |

Table 6.6.2.2.5.3-2: Additional requirements (network signalled value "NS_06" or "NS_07"), 3GHz < E-UTRA bands ≤ 4.2GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | Measurement bandwidth |
| 0-0.1 | -11.2 | -11.2 | -13.2 | -16.2 | 30 kHz |
| 0.1-1 | -11.2 | -11.2 | -11.2 | -11.2 | 100 kHz |
| 1-2.5 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 2.5-2.8 | -23.2 |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  | 1 MHz |
| 5-6 |  | -23.2 |  |  | 1 MHz |
| 6-10 |  |  | -23.2 |  | 1 MHz |
| 10-15 |  |  |  | -23.2 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.085 MHz. The first and last measurement position with a 100 kHz filter is at ΔfOOB equals to 0.15 MHz and 0.95 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_06 and NS_07 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |

NOTE:	(only for emission measurement) As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

6.6.2.2.5.4	Test requirements (network signalled value "NS_27")

When "NS_27" is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.4.5-27 as appropriate for a Power Class 3 UE

and

-	the power of any UE emission shall fulfil requirements in Table 6.6.2.2.5.4-1, as applicable.

Table 6.6.2.2.5.4-1: Additional requirements (network signalled value "NS_27")

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -11.2 | -11.2 | -11.2 | -11.2 | Note 1 |
| 1-10 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 10-15 |  | -23.2 | -23.2 | -23.2 | 1 MHz |
| 15-20 |  |  | -23.2 | -23.2 | 1 MHz |
| 20-25 |  |  |  | -23.2 | 1 MHz |
| Note 1: The measurement bandwidth is 1% of the applicable E-UTRA channel bandwidth. |  |  |  |  |  |

6.6.2.2.5.5	Test requirements (network signalled value "NS_35")

When "NS_35" is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.4.5-28 as appropriate for a Power Class 3 UE

and

-	the power of any UE emission shall fulfil requirements in Table 6.6.2.2.5.5-1, as applicable.

Table 6.6.2.2.5.5-1: Additional requirements (network signalled value "NS_35")

| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth (unless otherwise stated) |
| --- | --- | --- | --- | --- | --- |
| 0-0.1 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 0.1-6 | -11.5 | -11.5 | -11.5 | -11.5 | 100 kHz |
| 6-10 | -23.51 | -11.5 | -11.5 | -11.5 | 100 kHz |
| 10-15 |  | -23.51 | -11.5 | -11.5 | 100 kHz |
| 15-20 |  |  | -23.51 | -11.5 | 100 kHz |
| 20-25 |  |  |  | -23.5 | 1 MHz |
| Note 1: The measurement bandwidth shall be 1 MHz |  |  |  |  |  |

#### 6.6.2.2_1	Additional Spectrum Emission Mask for UL 64QAM

##### 6.6.2.2_1.1	Test purpose

Same test purpose as in clause 6.6.2.2.1

##### 6.6.2.2_1.2	Test applicability

This test case applies to all types of E-UTRA UE from release 13 and forward that support UL 64QAM.

##### 6.6.2.2_1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.6.2.2.3

##### 6.6.2.2_1.4	Test description

6.6.2.2_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Tables 6.6.2.2_1.4.1-1 through 6.6.2.2_1.4.1-9. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2 respectively. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.2_1.4.1-1: Test Configuration Table (network signalled value "NS_03")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Lowest, 5MHz, 10MHz, Highest |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |  |
| Ch BW | Mod'n | RB allocation |  | Mod'n | RB allocation |  |
|  |  | FDD | TDD |  | FDD | TDD |
| 1.4MHz | N/A for Additional Spectrum Emission Mask testing. |  |  | 64QAM | 5 | 5 |
| 3MHz |  |  |  | 64QAM | 15 (Note 3) | 15 |
| 3MHz |  |  |  | 64QAM | 4 | 4 |
| 5MHz |  |  |  | 64QAM | 25 (Note 3) | 25 |
| 5MHz |  |  |  | 64QAM | 8 | 8 |
| 10MHz |  |  |  | 64QAM | 50 (Note 3) | 50 |
| 10MHz |  |  |  | 64QAM | 12 | 12 |
| 15MHz |  |  |  | 64QAM | 75 (Note 3) | 75 |
| 15MHz |  |  |  | 64QAM | 16 | 16 |
| 20MHz |  |  |  | 64QAM | 100 (Note 3) | 100 |
| 20MHz |  |  |  | 64QAM | 18 | 18 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 3: The 64QAM configurations shall not apply to UEs for which TC 6.6.2.2_2 is executed with network signalled value NS_03. |  |  |  |  |  |  |

Table 6.6.2.2_1.4.1-2: Test Configuration Table (network signalled value "NS_04")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | Normal |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Low Range, Mid Range, High Range Additional test frequencies for Rel-12 to Rel-13 and bit 2 of modifiedMPRbehavior = 0: For 5 MHz bandwidth: 2499.6 MHz, NUL= 39686 For 10 MHz bandwidth: 2504.1 MHz, NUL= 39731 For 15 MHz bandwidth: 2510.9 MHz, NUL= 39799 For 20 MHz bandwidth: 2517.6 MHz, NUL= 39866 Additional test frequencies for Rel-12 and later and bit 2 of modifiedMPRbehavior = 1: For 5 MHz bandwidth: 2500.6 MHz, NUL= 39696 For 10 MHz bandwidth: 2504.1 MHz, NUL= 39731 For 15 MHz bandwidth: 2510.9 MHz, NUL= 39799 For 20 MHz bandwidth: 2517.6 MHz, NUL= 39866 |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 5MHz, 10MHz, 15MHz, 20MHz |  |  |
| Test Parameters for NS_04 A-MPR |  |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD | RBstart FDD |
| 1 (Note 1) | 5MHz | N/A for A-MPR testing |  | 64QAM | 16 | 9 |
| 2 (Note 2) | 5MHz |  |  | 64QAM | 25 | 0 |
| 3 | 10MHz |  |  | 64QAM | 50 | 0 |
| 4 | 15MHz |  |  | 64QAM | 75 | 0 |
| 5 | 20MHz |  |  | 64QAM | 100 | 0 |
| Note 1: Only applicable for Low Range. Note 2: Only applicable for Mid, High Range and additional test frequencies. Note 3: The 64QAM configurations shall not apply to UEs for which TC 6.6.2.2_2 is executed with network signalled value NS_04. |  |  |  |  |  |  |

Table 6.6.2.2_1.4.1-3: Test Configuration Table (network signalled value "NS_06")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Lowest, 5MHz, 10MHz |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |  |
| Ch BW | Mod'n | RB allocation |  | Mod'n | RB allocation |  |
|  |  | FDD | TDD |  | FDD | TDD |
| 1.4MHz | N/A for Additional Spectrum Emission |  |  | 64QAM | 5 | NA |
| 5MHz | Mask testing |  |  | 64QAM | 8 |  |
| 10MHz |  |  |  | 64QAM | 12 |  |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 3: The 64QAM configurations shall not apply to UEs for which TC 6.6.2.2_2 is executed with network signalled value NS_06. |  |  |  |  |  |  |

Table 6.6.2.2_1.4.1-4: Test Configuration Table (network signalled value "NS_07")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  |  |  | Mid range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 10MHz |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Test Number | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD | RBstart |
| 1 | 10MHz | N/A for Additional Spectrum Emission Mask testing. |  | 64QAM | 36 | 13 |
| 2 | 10MHz |  |  | 64QAM | 16 | 19 |
| 3 | 10MHz |  |  | 64QAM | 30 | 19 |
| 4 | 10MHz |  |  | 64QAM | 50 | 0 |
| Note 1: The 64QAM configurations shall not apply to UEs for which TC 6.6.2.2_2 is executed with network signalled value NS_07. |  |  |  |  |  |  |

Table 6.6.2.2_1.4.1-5: Test Configuration Table (network signalled value "NS_11")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | Normal |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range For 3 MHz Channel Bandwidth: a. 2001.5 MHz (NUL = 25515) b. 2004.5 MHz (NUL = 25545) For 5 MHz Channel Bandwidth a. 2002.5 MHz (NUL = 25525) b. 2004.5 MHz (NUL = 25545) c. 2007.5 MHz (NUL = 25575) For 10 MHz Channel Bandwidth a. 2005 MHz (NUL = 25550) b. 2005.5 MHz (NUL = 25555) c. 2015 MHz (NUL = 25650) For 15 MHz Channel Bandwidth a. 2007.5 MHz (NUL = 25575) b. 2012.5 MHz (NUL = 25625) For 20 MHz Channel Bandwidth a. 2010 MHz (NUL = 25600) |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 3MHz, 5MHz, 10MHz, 15MHz, 20MHz |  |
| Test Parameters for NS_11 A-MPR |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |
| Configuration ID | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD |
| 1 | 3MHz | N/A for Additional Spectrum Emission Mask testing. |  | 64QAM | 6 |
| 2 | 3MHz |  |  | 64QAM (Note 3) | 15 |
| 3 | 5MHz |  |  | 64QAM | 8 |
| 4 | 5MHz |  |  | 64QAM (Note 3) | 25 |
| 5 | 10MHz |  |  | 64QAM | 12 |
| 6 | 10MHz |  |  | 64QAM (Note 3) | 50 |
| 7 | 15MHz |  |  | 64QAM (Note 3) | 8 |
| 8 | 15MHz |  |  | 64QAM (Note 3) | 25 |
| 9 | 15MHz |  |  | 64QAM (Note 3) | 30 |
| 10 | 15MHz |  |  | 64QAM (Note 3) | 75 |
| 11 | 20MHz |  |  | 64QAM (Note 3) | 10 |
| 12 | 20MHz |  |  | 64QAM (Note 3) | 25 |
| 13 | 20MHz |  |  | 64QAM (Note 3) | 100 |
| Note 1: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4.5 as not all combinations are necessarily required based on the applicability of the UE. Note 2: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 3: The 64QAM configurations shall not apply to UEs for which TC 6.6.2.2_2 is executed with network signalled value NS_11. |  |  |  |  |  |

Table 6.6.2.2_1.4.1-6: Test Configuration Table (network signalled value "NS_20")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | Normal |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range For 5 MHz Channel Bandwidth a. 2002.5 MHz (NUL = 25525) b. 2007.5 MHz (NUL = 25575) c. 2012.5 MHz (NUL = 25625) d. 2017.5 MHz (NUL = 25675) For 10 MHz Channel Bandwidth a. 2005 MHz (NUL = 25550) b. 2015 MHz (NUL = 25650) For 15 MHz Channel Bandwidth a. 2012.5 MHz (NUL = 25625) For 20 MHz Channel Bandwidth a. 2010 MHz (NUL = 25600) |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 5MHz, 10MHz, 15MHz, 20MHz |  |
| Test Parameters for NS_20 A-MPR |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |
| Configuration ID | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD |
| 1 | 5MHz | N/A for Additional Spectrum Emission Mask testing. |  | 64QAM (Note 3) | 15 |
| 2 | 5MHz |  |  | 64QAM (Note 3) | 25 |
| 3 | 10MHz |  |  | 64QAM | 12 |
| 4 | 10MHz |  |  | 64QAM (Note 3) | 50 |
| 5 | 15MHz |  |  | 64QAM (Note 3) | 25 |
| 6 | 15MHz |  |  | 64QAM (Note 3) | 36 |
| 7 | 15MHz |  |  | 64QAM (Note 3) | 75 |
| 8 | 20MHz |  |  | 64QAM | 18 |
| 9 | 20MHz |  |  | 64QAM (Note 3) | 25 |
| 10 | 20MHz |  |  | 64QAM (Note 3) | 75 |
| 11 | 20MHz |  |  | 64QAM (Note 3) | 100 |
| Note 1: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4.5 as not all combinations are necessarily required based on the applicability of the UE. Note 2: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 3: The 64QAM configurations shall not apply to UEs for which TC 6.6.2.2_2 is executed with network signalled value NS_20. |  |  |  |  |  |

Table 6.6.2.2_1.4.1-7: Test Configuration Table (network signalled value "NS_21")

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  |  |  | NC |  |  |  |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  |  |  |  |  |  | High range |  |  |  |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Downlink Configuration |  |  | Uplink Configuration |  |  |  |  |  |
| Configuration ID |  | Ch BW |  | Mod'n |  | RB allocation | Mod'n |  | RB allocation FDD |  | RBstart FDD |  |
| 1 |  | 10 MHz |  | N/A for Additional Spectrum Emission Mask testing. |  |  | 64QAM |  | 50 |  | 0 |  |
| Note 1: The 64QAM configurations shall not apply to UEs for which TC 6.6.2.2_2 is executed with network signalled value NS_21. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.6.2.2_1.4.1-8: Test Configuration Table (network signalled value "NS_27")

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  | Normal |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | Refer to Frequency column in test parameters |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | 5MHz, 10MHz, 15MHz, 20MHz |  |  |
| Test Parameters for NS_27 A-MPR |  |  |  |  |  |  |  |
|  |  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID | Frequency | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD | RBstart FDD |
| 1 | Low/Mid/High | 5MHz |  |  | 64QAM | 25 | 0 |
| 2 | Low/Mid/High | 10MHz |  |  | 64QAM | 50 | 0 |
| 3 | Low/Mid/High | 15MHz |  |  | 64QAM | 75 | 0 |
| 4 | Low/Mid/High | 20MHz |  |  | 64QAM | 100 | 0 |
| Note 1: The 64QAM configurations shall not apply to UEs for which TC 6.6.2.2_2 is executed with network signalled value NS_27. |  |  |  |  |  |  |  |

Table 6.6.2.2_1.4.1-9: Test Configuration Table (network signalled value "NS_35")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 5MHz, 10MHz, 15MHz, 20MHz |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |  |
| Ch BW | Mod'n | RB allocation |  | Mod'n | RB allocation |  |
|  |  | FDD | TDD |  | FDD | TDD |
| 5MHz | N/A for Additional Spectrum Emission Mask testing. |  |  | 64QAM | 8 | NA |
| 10MHz |  |  |  | 64QAM | 12 |  |
| 15MHz |  |  |  | 64QAM | 16 |  |
| 20MHz |  |  |  | 64QAM | 18 |  |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 3: The 64QAM configurations shall not apply to UEs for which TC 6.6.2.2_2 is executed with network signalled value NS_35. |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.3.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The DL Reference Measurement channels are set according to Tables 6.6.2.2_1.4.1-1 to 6.6.2.2_1.4.1-9.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.2_1.4.3.

6.6.2.2_1.4.2	Test procedure

Same test procedure as in clause 6.6.2.2.4.2

6.6.2.2_1.4.3	Message contents

Same message contents as in clause 6.6.2.2.4.3

##### 6.6.2.2_1.5	Test requirements

Same test requirements as in clause 6.6.2.2.5

#### 6.6.2.2_2	Additional Spectrum Emission Mask for UL 256QAM

##### 6.6.2.2_2.1	Test purpose

Same test purpose as in clause 6.6.2.2.1

##### 6.6.2.2_2.2	Test applicability

This test case applies to all types of E-UTRA UE from release 14 and forward that support UL 256QAM.

##### 6.6.2.2_2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.6.2.2.3

##### 6.6.2.2_2.4	Test description

6.6.2.2_2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Tables 6.6.2.2_2.4.1-1 through 6.6.2.2_2.4.1-9. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2 respectively. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.2_2.4.1-1: Test Configuration Table (network signalled value "NS_03")

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | Lowest, 5MHz, 10MHz, Highest |  |  |
| Test Parameters for NS_03 A-MPR |  |  |  |  |  |  |  |
|  |  | Downlink Configuration |  |  | Uplink Configuration |  |  |
| Configura-tion ID | Ch BW | Mod'n | RB allocation |  | Mod'n | RB allocation |  |
|  |  |  | FDD | TDD |  | FDD | TDD |
| 1 | 3MHz | N/A for Additional Spectrum Emission Mask testing |  |  | 256QAM | 15 | 15 |
| 2 | 5MHz |  |  |  | 256QAM | 25 | 25 |
| 3 | 10MHz |  |  |  | 256QAM | 50 | 50 |
| 4 | 15MHz |  |  |  | 256QAM | 75 | 75 |
| 5 | 20MHz |  |  |  | 256QAM | 100 | 100 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. |  |  |  |  |  |  |  |

Table 6.6.2.2_2.4.1-2: Test Configuration Table (network signalled value "NS_04")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | Normal |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Low Range, Mid Range, High Range Additional test frequencies for Rel-12 to Rel-13 and bit 2 of modifiedMPRbehavior = 0: For 5 MHz bandwidth: 2499.6 MHz, NUL= 39686 For 10 MHz bandwidth: 2504.1 MHz, NUL= 39731 For 15 MHz bandwidth: 2510.9 MHz, NUL= 39799 For 20 MHz bandwidth: 2517.6 MHz, NUL= 39866 Additional test frequencies for Rel-12 and later and bit 2 of modifiedMPRbehavior = 1: For 5 MHz bandwidth: 2500.6 MHz, NUL= 39696 For 10 MHz bandwidth: 2504.1 MHz, NUL= 39731 For 15 MHz bandwidth: 2510.9 MHz, NUL= 39799 For 20 MHz bandwidth: 2517.6 MHz, NUL= 39866 |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 5MHz, 10MHz, 15MHz, 20MHz |  |  |
| Test Parameters for NS_04 A-MPR |  |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD | RBstart FDD |
| 1 (Note 1) | 5MHz | N/A for Additional Spectrum Emission Mask testing |  | 256QAM | 16 | 9 |
| 2 (Note 2) | 5MHz |  |  | 256QAM | 25 | 0 |
| 3 | 10MHz |  |  | 256QAM | 50 | 0 |
| 4 | 15MHz |  |  | 256QAM | 75 | 0 |
| 5 | 20MHz |  |  | 256QAM | 100 | 0 |
| Note 1: Only applicable for Low Range. Note 2: Only applicable for Mid, High Range and additional test frequencies. |  |  |  |  |  |  |

Table 6.6.2.2_2.4.1-3: Test Configuration Table (network signalled value "NS_06")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | Normal |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Lowest, 5MHz, 10MHz, Highest |  |
| Test Parameters for NS_06 A-MPR |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |
| Configura-tion ID | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD |
| 1 | 1.4MHz | N/A for Additional Spectrum Emission Mask testing |  | 256QAM | 6 |
| 2 | 5MHz |  |  | 256QAM | 25 |
| 3 | 10MHz |  |  | 256QAM | 50 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. |  |  |  |  |  |

Table 6.6.2.2_2.4.1-4: Test Configuration Table (network signalled value "NS_07")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Mid range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 10MHz |  |  |
| Test Parameters for NS_07 A-MPR |  |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configura-tion ID | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD | RBstart FDD |
| 1 | 10MHz | N/A for Additional Spectrum Emission Mask testing |  | 256QAM | 36 | 13 |
| 2 | 10MHz |  |  | 256QAM | 16 | 19 |
| 3 | 10MHz |  |  | 256QAM | 30 | 19 |
| 4 | 10MHz |  |  | 256QAM | 50 | 0 |

Table 6.6.2.2_2.4.1-5: Test Configuration Table (network signalled value "NS_11")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | Normal |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | For 3 MHz Channel Bandwidth: a. 2001.5 MHz (NUL = 25515) b. 2004.5 MHz (NUL = 25545) For 5 MHz Channel Bandwidth a. 2002.5 MHz (NUL = 25525) b. 2004.5 MHz (NUL = 25545) c. 2007.5 MHz (NUL = 25575) For 10 MHz Channel Bandwidth a. 2005 MHz (NUL = 25550) b. 2005.5 MHz (NUL = 25555) c. 2015 MHz (NUL = 25650) For 15 MHz Channel Bandwidth a. 2007.5 MHz (NUL = 25575) b. 2012.5 MHz (NUL = 25625) For 20 MHz Channel Bandwidth a. 2010 MHz (NUL = 25600) |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 3MHz, 5MHz, 10MHz, 15MHz, 20MHz |  |
| Test Parameters for NS_11 A-MPR |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |
| Configuration ID | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD |
| 1 | 3MHz | N/A for Additional Spectrum Emission Mask testing |  | 256QAM | 15 |
| 2 | 5MHz |  |  | 256QAM | 25 |
| 3 | 10MHz |  |  | 256QAM | 50 |
| 4 | 15MHz |  |  | 256QAM | 8 |
| 5 | 15MHz |  |  | 256QAM | 25 |
| 6 | 15MHz |  |  | 256QAM | 30 |
| 7 | 15MHz |  |  | 256QAM | 75 |
| 8 | 20MHz |  |  | 256QAM | 10 |
| 9 | 20MHz |  |  | 256QAM | 25 |
| 10 | 20MHz |  |  | 256QAM | 100 |
| Note 1: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.6.2.2_2.5 as not all combinations are necessarily required based on the applicability of the UE. Note 2: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. |  |  |  |  |  |

Table 6.6.2.2_2.4.1-6: Test Configuration Table (network signalled value "NS_20")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | Normal |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | For 5 MHz Channel Bandwidth a. 2002.5 MHz (NUL = 25525) b. 2007.5 MHz (NUL = 25575) c. 2012.5 MHz (NUL = 25625) d. 2017.5 MHz (NUL = 25675) For 10 MHz Channel Bandwidth a. 2005 MHz (NUL = 25550) b. 2015 MHz (NUL = 25650) For 15 MHz Channel Bandwidth a. 2012.5 MHz (NUL = 25625) For 20 MHz Channel Bandwidth a. 2010 MHz (NUL = 25600) |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 5MHz, 10MHz, 15MHz, 20MHz |  |
| Test Parameters for NS_20 A-MPR |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |
| Configuration ID | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD |
| 1 | 5MHz | N/A for Additional Spectrum Emission Mask testing |  | 256QAM | 15 |
| 2 | 5MHz |  |  | 256QAM | 25 |
| 3 | 10MHz |  |  | 256QAM | 50 |
| 4 | 15MHz |  |  | 256QAM | 25 |
| 5 | 15MHz |  |  | 256QAM | 36 |
| 6 | 15MHz |  |  | 256QAM | 75 |
| 7 | 20MHz |  |  | 256QAM | 25 |
| 8 | 20MHz |  |  | 256QAM | 75 |
| 9 | 20MHz |  |  | 256QAM | 100 |
| Note 1: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.6.2.2_2.5 as not all combinations are necessarily required based on the applicability of the UE. Note 2: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. |  |  |  |  |  |

Table 6.6.2.2_2.4.1-7: Test Configuration Table (network signalled value "NS_21")

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  |  |  | NC |  |  |  |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  |  |  | High range |  |  |  |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |
| Test Parameters for NS_21 A-MPR |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Downlink Configuration |  |  | Uplink Configuration |  |  |  |  |  |
| Configuration ID |  | Ch BW |  | Mod'n |  | RB allocation | Mod'n |  | RB allocation FDD |  | RBstart FDD |  |
| 1 |  | 10 MHz |  | N/A for Additional Spectrum Emission Mask testing |  |  | 256QAM |  | 50 |  | 0 |  |

Table 6.6.2.2_2.4.1-8: Test Configuration Table (network signalled value "NS_27")

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  | Normal |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | Refer to Frequency column in test parameters |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | 5MHz, 10MHz, 15MHz, 20MHz |  |  |
| Test Parameters for NS_27 A-MPR |  |  |  |  |  |  |  |
|  |  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID | Frequency | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD | RBstart FDD |
| 1 | Low/Mid/High | 5MHz | N/A for Additional Spectrum Emission Mask testing |  | 256QAM | 25 | 0 |
| 2 | Low/Mid/High | 10MHz |  |  | 256QAM | 50 | 0 |
| 3 | Low/Mid/High | 15MHz |  |  | 256QAM | 75 | 0 |
| 4 | Low/Mid/High | 20MHz |  |  | 256QAM | 100 | 0 |

Table 6.6.2.2_2.4.1-9: Test Configuration Table (network signalled value "NS_35")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | Normal |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 5MHz, 10MHz, 15MHz, 20MHz |  |
| Test Parameters for NS_35 A-MPR |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |
| Configura-tion ID | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD |
| 1 | 5MHz | N/A for Additional Spectrum Emission Mask testing |  | 256QAM | 25 |
| 2 | 10MHz |  |  | 256QAM | 50 |
| 3 | 15MHz |  |  | 256QAM | 75 |
| 4 | 20MHz |  |  | 256QAM | 100 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.3.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The DL Reference Measurement channels are set according to Tables 6.6.2.2_2.4.1-1 to 6.6.2.2_2.4.1-9.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.2_2.4.3.

6.6.2.2_2.4.2	Test procedure

Same test procedure as in clause 6.6.2.2.4.2

6.6.2.2_2.4.3	Message contents

Same message contents as in clause 6.6.2.2.4.3

##### 6.6.2.2_2.5	Test requirements

Same test requirements as in clause 6.6.2.2.5

#### 6.6.2.2_3	Additional Spectrum Emission Mask with PUSCH frequency hopping

Editor’s notes: This test case is incomplete. The following aspects are either missing or TBD.

-	Annex F is not updated

-	HARQ configuration for re-transmissions is missing

6.6.2.2_3.1	Test purpose

To verify that the power of any UE emission shall not exceed specified level for the specified channel bandwidth under the deployment scenarios where additional requirements are specified.

6.6.2.2_3.2	Test applicability

This test case applies to all types of E-UTRA UE release 8 and forward.

6.6.2.2_3.3	Minimum conformance requirements

Same minimum conformance requirements as section 6.6.2.2.3

6.6.2.2_3.4	Test description

6.6.2.2_3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 6.6.2.2_3.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2 respectively. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.2_3.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  | Normal |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | Mid range |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | Test dependant |  |
| Test Parameters |  |  |  |  |  |  |
|  |  |  | Downlink Configuration |  | Uplink Configuration |  |
| Configuration ID | NS-value | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD |
| 1 | NS_07 | 10 MHz | N/A for A-MPR testing |  | QPSK | 1 |
| 3 (Note 3) | NS_11 | 15 MHz |  |  | QPSK | 1 |
| 10 (Note 4) | NS_20 | 10 MHz |  |  | QPSK | 1 |
| Note 1: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4_3.5 as not all combinations are necessarily required based on the applicability of the UE. Note 2: The RBstart of partial RB allocation shall be RB# 0 of the channel bandwidth. Note 3: High range test frequency to be used. Note 4: Low range test frequency to be used. |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.3.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The DL Reference Measurement channels are set according to Table 6.6.2.2_3.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.2.4.3.

6.6.2.2_3.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 with hopping bit(s) set to 1 (PUSCH type 2 hopping) for C_RNTI to schedule the UL RMC according to the applicable Table 6.6.2.2_3.4.1-1. SS sends always NACK on PHICH to trigger UE retransmissions needed for mirroring of RBs. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode in a TTI. The period of measurement shall be one sub-frame (1ms) excluding the 20 us transient periods in beginning and end of the subframe.

4.	Measure the mean power of the UE in the channel bandwidth of the radio access mode in the next TTI of the same HARQ process. The period of measurement shall be one sub-frame (1ms) excluding the 20 us transient periods in beginning and end of the subframe.

5.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to Tables 6.6.2.2.5-1 to 6.6.2.2.5-6 and using a rms detector, as applicable. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs excluding the 20 us transient periods in beginning and end of the subframe. The measurement period shall include at least two consecutive UL TTI’s.

6.6.2.2_3.4.3	Message contents

Message content is the same as in section 6.6.2.2.4.2

6.6.2.2_3.5	Test requirements

-	the measured UE mean power in the channel bandwidth, derived in step 3 and 4, shall fulfil requirements in Table 6.2.4_3.5-1 and 6.2.4_3.5-2 as appropriate,

and

-	the power of any UE emission shall fulfil requirements in Table 6.6.2.2_3.5-1 to 6.6.2.2_3.5-6, as applicable.

Table 6.6.2.2_3.5-1: Additional requirements (network signalled value "NS_03", “NS_11”, “NS_20” and “NS_21”), E-UTRA bands ≤ 3GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.5 | -11.5 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 1-2.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 |  |  |  |  | 1 MHz |
| 6-10 |  |  | -23.5 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.5 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.5 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_03 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

Table 6.6.2.2_3.5-2: Additional requirements (network signalled value "NS_03", “NS_11”, “NS_20” and “NS_21”), 3GHz < E-UTRA bands ≤ 4.2GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.2 | -11.2 | -13.2 | -16.2 | -18.2 | -19.2 | 30 kHz |
| 1-2.5 | -11.2 | -11.2 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 2.5-2.8 | -23.2 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.2 |  |  |  |  | 1 MHz |
| 6-10 |  |  | -23.2 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.2 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.2 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.2 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_03 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

NOTE:	(only for emission measurement) As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

Table 6.6.2.2_3.5-3: Additional requirements (network signalled value "NS_04"), E-UTRA bands ≤ 3GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.5 | -11.5 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 1-2.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 | -23.5 | -23.5 | -23.5 | -23.5 | 1 MHz |
| 6-10 |  |  |  |  |  |  | 1 MHz |
| 10-15 |  |  |  |  |  |  | 1 MHz |
| 15-20 |  |  |  |  |  |  | 1 MHz |
| 20-25 |  |  |  |  |  |  | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_04 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

Table 6.6.2.2_3.5-4: Additional requirements (network signalled value "NS_04"), 3GHz < E-UTRA bands ≤ 4.2GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.2 | -11.2 | -13.2 | -16.2 | -18.2 | -19.2 | 30 kHz |
| 1-2.5 | -11.2 | -11.2 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 2.5-2.8 | -23.2 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.2 | -23.2 | -23.2 | -23.2 | -23.2 | 1 MHz |
| 6-10 |  |  |  |  |  |  | 1 MHz |
| 10-15 |  |  |  |  |  |  | 1 MHz |
| 15-20 |  |  |  |  |  |  | 1 MHz |
| 20-25 |  |  |  |  |  |  | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_04 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

NOTE:	(only for emission measurement) As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

Table 6.6.2.2_3.5-5: Additional requirements (network signalled value "NS_06" or "NS_07"), E-UTRA bands ≤ 3GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | Measurement bandwidth |
| 0-0.1 | -11.5 | -11.5 | -13.5 | -16.5 | 30 kHz |
| 0.1-1 | -11.5 | -11.5 | -11.5 | -11.5 | 100 kHz |
| 1-2.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 |  |  | 1 MHz |
| 6-10 |  |  | -23.5 |  | 1 MHz |
| 10-15 |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.085 MHz. The first and last measurement position with a 100 kHz filter is at ΔfOOB equals to 0.15 MHz and 0.95 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_06 and NS_07 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |

Table 6.6.2.2_3.5-6: Additional requirements (network signalled value "NS_06" or "NS_07"), 3GHz < E-UTRA bands ≤ 4.2GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | Measurement bandwidth |
| 0-0.1 | -11.2 | -11.2 | -13.2 | -16.2 | 30 kHz |
| 0.1-1 | -11.2 | -11.2 | -11.2 | -11.2 | 100 kHz |
| 1-2.5 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 2.5-2.8 | -23.2 |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  | 1 MHz |
| 5-6 |  | -23.2 |  |  | 1 MHz |
| 6-10 |  |  | -23.2 |  | 1 MHz |
| 10-15 |  |  |  | -23.2 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.085 MHz. The first and last measurement position with a 100 kHz filter is at ΔfOOB equals to 0.15 MHz and 0.95 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_06 and NS_07 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |

NOTE:	(only for emission measurement) As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

#### 6.6.2.2A	Additional Spectrum Emission Mask for CA

##### 6.6.2.2A.1	Additional Spectrum Emission Mask for CA (intra-band contiguous DL CA and UL CA)

6.6.2.2A.1.1	Test purpose

To verify that the power of any UE emission shall not exceed specified lever for the specified channel bandwidth for CA under the deployment scenarios where additional requirements are specified.

6.6.2.2A.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 10 and forward that support intra-band contiguous DL CA and UL CA but do not support UL 64QAM or UL 256QAM.

6.6.2.2A.1.3	Minimum conformance requirements

6.6.2.2A.1.3.1	Minimum requirements (network signalled value "CA_NS_04")

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When "CA_NS_04" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.2.2A.1.3.1-1.

Table 6.6.2.2A.1.3.1-1: Additional requirements

| Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 50+75RB (24.75 MHz) | 25+100RB (24.95 MHz) | 50+100RB (29.9 MHz) | 75+75RB (30 MHz) | 75+100RB (34.85 MHz) | 100+100RB (39.8 MHz) | Measurement bandwidth |
| 0-1 | -22 | -22 | -22.5 | -23 | -23.5 | -24 | 30 kHz |
| 1-5 | -10 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 5-22.95 | -13 | -13 | -13 | -13 | -13 | -13 | 1 MHz |
| 22.95-23.25 | -13 | -25 | -13 | -13 | -13 | -13 | 1 MHz |
| 23.25-27.9 | -25 | -25 | -13 | -13 | -13 | -13 | 1 MHz |
| 27.9-28.5 | -25 | -25 | -25 | -13 | -13 | -13 | 1 MHz |
| 28.5-29.75 | -25 | -25 | -25 | -25 | -13 | -13 | 1 MHz |
| 29.75-29.95 |  | -25 | -25 | -25 | -13 | -13 | 1 MHz |
| 29.95-32.85 |  |  | -25 | -25 | -13 | -13 | 1 MHz |
| 32.85-34.9 |  |  | -25 | -25 | -25 | -13 | 1 MHz |
| 34.9-35 |  |  |  | -25 | -25 | -13 | 1 MHz |
| 35-37.8 |  |  |  |  | -25 | -13 | 1 MHz |
| 37.8-39.85 |  |  |  |  | -25 | -25 | 1 MHz |
| 39.85-44.8 |  |  |  |  |  | -25 | 1 MHz |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

The normative reference for this requirement is TS 36.101[2] subclause 6.6.2.2A.

6.6.2.2A.1.4	Test description

6.6.2.2A.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.2A.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.2A.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  |  | C: Low and High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  |  | As in Table 6.6.2.2A.1.3.1-1 |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |  |
|  | PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc |  | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 100 | 50 | N/A | QPSK | 10 |  | P_10@20 | S_0@0 |  |  |
| 2 | 100 | 50 |  | QPSK | 60 |  | P_50@50 | S_10@0 |  |  |
| 3 | 100 | 50 |  | 16QAM | 15 |  | P_0@0 | S_15@0 |  |  |
| 4 | 100 | 50 |  | QPSK | 2 |  | P_1@0 | S_1@49 |  |  |
| 5 | 75 | 75 |  | QPSK | 10 |  | P_10@20 | S_0@0 |  |  |
| 6 | 75 | 75 |  | QPSK | 75 |  | P_30@45 | S_45@0 |  |  |
| 7 | 75 | 75 |  | QPSK | 2 |  | P_1@0 | S_1@74 |  |  |
| 8 | 100 | 75 |  | QPSK | 10 |  | P_10@20 | S_0@0 |  |  |
| 9 | 100 | 75 |  | QPSK | 80 |  | P_50@50 | S_30@0 |  |  |
| 10 | 100 | 75 |  | 16QAM | 20 |  | P_0@0 | S_20@15 |  |  |
| 11 | 100 | 75 |  | QPSK | 2 |  | P_1@0 | S_1@74 |  |  |
| 12 | 100 | 100 |  | QPSK | 10 |  | P_10@25 | S_0@0 |  |  |
| 13 | 100 | 100 |  | QPSK | 90 |  | P_40@60 | S_50@0 |  |  |
| 14 | 100 | 100 |  | 16QAM | 15 |  | P_0@0 | S_15@40 |  |  |
| 15 | 100 | 100 |  | 16QAM | 20 |  | P_0@0 | S_20@30 |  |  |
| 16 | 100 | 100 |  | QPSK | 2 |  | P_1@0 | S_1@99 |  |  |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1 |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] clause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.2A.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.2A.1.4.3.

6.6.2.2A.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.6.2.2A.1.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.6.2.2A.1.4.1-1 on both PCC and SCC. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

6.	Measure the mean power over all component carriers in the CA configuration of the radio access mode, which shall meet the requirements described in Table 6.2.4A.1.5-4. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

7.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to table 6.6.2.2A.1.5.1-1 and using a rms detector. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.6.2.2A.1.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6. The following exceptions apply for each network signalled value.

6.6.2.2A.1.4.3.1	Message contents exceptions (network signalled value "CA_NS_04")

1.	Information element additionalSpectrumEmissionSCell-r10 is set to CA_NS_04. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages. This exception indicates that the UE shall meet the additional spectrum emission requirement for a specific deployment scenario.

Table 6.6.2.2A.1.4.3.1-1: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "CA_NS_04"

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 4 (CA_NS_04) |  |  |

6.6.2.2A.1.5	Test Requirements

6.6.2.2A.1.5.1	Test requirement for CA (network signalled value "CA_NS_04")

When "CA_NS_04" is indicated in the cell

-	the measured UE mean power with the aggregated channel bandwidth as specified in clause 5.4.2A, derived in step 6 , shall fulfil requirements in Table 6.2.4A.1.5-4 as appropriate,

and

-	the power of any UE emission shall not exceed the levels specified in Table 6.6.2.2A.1.5.1-1.

Table 6.6.2.2A.1.5.1-1: Additional requirements

| Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 50+75RB (24.75 MHz) | 25+100RB (24.95 MHz) | 50+100RB (29.9 MHz) | 75+75RB (30 MHz) | 75+100RB (34.85 MHz) | 100+100RB (39.8 MHz) | Measurement bandwidth |
| 0-1 | -22 | -22 | -22.5 | -23 | -23.5 | -24 | 30 kHz |
| 1-5 | -10 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 5-22.95 | -13 | -13 | -13 | -13 | -13 | -13 | 1 MHz |
| 22.95-23.25 | -13 | -25 | -13 | -13 | -13 | -13 | 1 MHz |
| 23.25-27.9 | -25 | -25 | -13 | -13 | -13 | -13 | 1 MHz |
| 27.9-28.5 | -25 | -25 | -25 | -13 | -13 | -13 | 1 MHz |
| 28.5-29.75 | -25 | -25 | -25 | -25 | -13 | -13 | 1 MHz |
| 29.75-29.95 |  | -25 | -25 | -25 | -13 | -13 | 1 MHz |
| 29.95-32.85 |  |  | -25 | -25 | -13 | -13 | 1 MHz |
| 32.85-34.9 |  |  | -25 | -25 | -25 | -13 | 1 MHz |
| 34.9-35 |  |  |  | -25 | -25 | -13 | 1 MHz |
| 35-37.8 |  |  |  |  | -25 | -13 | 1 MHz |
| 37.8-39.85 |  |  |  |  | -25 | -25 | 1 MHz |
| 39.85-44.8 |  |  |  |  |  | -25 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel. Note 4: Above SEM requirement applies to bands corresponding to network signalling value CA_NS_04 as defined in TS 36.101 [2] subclause 6.6.2.2A Table 6.2.2.2A-1. |  |  |  |  |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

##### 6.6.2.2A.1_1	Additional Spectrum Emission Mask for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM

Editor's note: For a transition period until RAN5#83, this test case in version 15.3.1 of 36.521-1 shall be used. This is to ensure no test coverage is lost before the UL 256QAM test case becomes available.

6.6.2.2A.1_1.1	Test purpose

Same test purpose as in clause 6.6.2.2A.1.1.

6.6.2.2A.1_1.2	Test applicability

This test case applies to all types of E-UTRA UE from release 13 and forward that support intra-band contiguous DL CA, UL CA and UL 64QAM but do not support UL 256QAM.

6.6.2.2A.1_1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.6.2.2.A.1.3.

6.6.2.2A.1_1.4	Test description

6.6.2.2A.1_1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.2A.1_1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.2A.1_1.4.1-1: Test Configuration Table for CA_NS_04

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  |  | C: Low and High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  |  | As in Table 6.6.2.2A.1.3.1-1 |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |  |
|  | PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc |  | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 100 | 50 | N/A | 64QAM | 10 |  | P_10@20 | S_0@0 |  |  |
| 2 | 100 | 50 |  | 64QAM | 60 |  | P_50@50 | S_10@0 |  |  |
| 3 | 100 | 50 |  | 64QAM | 15 |  | P_0@0 | S_15@0 |  |  |
| 4 | 100 | 50 |  | 64QAM | 2 |  | P_1@0 | S_1@49 |  |  |
| 5 | 75 | 75 |  | 64QAM | 10 |  | P_10@20 | S_0@0 |  |  |
| 6 | 75 | 75 |  | 64QAM | 75 |  | P_30@45 | S_45@0 |  |  |
| 7 | 75 | 75 |  | 64QAM | 2 |  | P_1@0 | S_1@74 |  |  |
| 8 | 100 | 75 |  | 64QAM | 10 |  | P_10@20 | S_0@0 |  |  |
| 9 | 100 | 75 |  | 64QAM | 80 |  | P_50@50 | S_30@0 |  |  |
| 10 | 100 | 75 |  | 64QAM | 20 |  | P_0@0 | S_20@15 |  |  |
| 11 | 100 | 75 |  | 64QAM | 2 |  | P_1@0 | S_1@74 |  |  |
| 12 | 100 | 100 |  | 64QAM | 10 |  | P_10@25 | S_0@0 |  |  |
| 13 | 100 | 100 |  | 64QAM | 90 |  | P_40@60 | S_50@0 |  |  |
| 14 | 100 | 100 |  | 64QAM | 15 |  | P_0@0 | S_15@40 |  |  |
| 15 | 100 | 100 |  | 64QAM | 20 |  | P_0@0 | S_20@30 |  |  |
| 16 | 100 | 100 |  | 64QAM | 2 |  | P_1@0 | S_1@99 |  |  |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1 |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] clause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.2A.1_1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.2A.1_1.4.3.

6.6.2.2A.1_1.4.2	Test procedure

Same test procedure as in clause 6.6.2.2A.1.4.2 with following exceptions:

-	Instead of Table 6.2.4A.1.5-4 use Table 6.2.4A.1_1.5-4.

6.6.2.2A.1_1.4.3	Message contents

Same message contents as in clause 6.6.2.2A.1.4.3.

6.6.2.2A.1_1.5	Test Requirements

Same test requirements as in clause 6.6.2.2A.1.5 with following exceptions:

-	Instead of Table 6.2.4A.1.5-4 use Table 6.2.4A.1_1.5-4.

##### 6.6.2.2A.1_2	Additional Spectrum Emission Mask for CA (intra-band contiguous DL CA and UL CA) for UL 256QAM

6.6.2.2A.1_2.1	Test purpose

Same test purpose as in clause 6.6.2.2A.1.1.

6.6.2.2A.1_2.2	Test applicability

This test case applies to all types of E-UTRA UE release 14 and forward that support intra-band contiguous DL CA and UL CA and UL 256QAM.

6.6.2.2A.1_2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.6.2.2A.1.3.

6.6.2.2A.1_2.4	Test description

6.6.2.2A.1_2.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.2A.1_2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.2A.1_2.4.1-1: Test Configuration Table for CA_NS_04

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  |  | C: Low and High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  |  | As in Table 6.6.2.2A.1.3.1-1 |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |  |
|  | PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc |  | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 100 | 50 | N/A | 256QAM | 10 |  | P_10@20 | S_0@0 |  |  |
| 2 | 100 | 50 |  | 256QAM | 60 |  | P_50@50 | S_10@0 |  |  |
| 3 | 100 | 50 |  | 256QAM | 15 |  | P_0@0 | S_15@0 |  |  |
| 4 | 100 | 50 |  | 256QAM | 2 |  | P_1@0 | S_1@49 |  |  |
| 5 | 75 | 75 |  | 256QAM | 10 |  | P_10@20 | S_0@0 |  |  |
| 6 | 75 | 75 |  | 256QAM | 75 |  | P_30@45 | S_45@0 |  |  |
| 7 | 75 | 75 |  | 256QAM | 2 |  | P_1@0 | S_1@74 |  |  |
| 8 | 100 | 75 |  | 256QAM | 10 |  | P_10@20 | S_0@0 |  |  |
| 9 | 100 | 75 |  | 256QAM | 80 |  | P_50@50 | S_30@0 |  |  |
| 10 | 100 | 75 |  | 256QAM | 20 |  | P_0@0 | S_20@15 |  |  |
| 11 | 100 | 75 |  | 256QAM | 2 |  | P_1@0 | S_1@74 |  |  |
| 12 | 100 | 100 |  | 256QAM | 10 |  | P_10@25 | S_0@0 |  |  |
| 13 | 100 | 100 |  | 256QAM | 90 |  | P_40@60 | S_50@0 |  |  |
| 14 | 100 | 100 |  | 256QAM | 15 |  | P_0@0 | S_15@40 |  |  |
| 15 | 100 | 100 |  | 256QAM | 20 |  | P_0@0 | S_20@30 |  |  |
| 16 | 100 | 100 |  | 256QAM | 2 |  | P_1@0 | S_1@99 |  |  |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1 |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] clause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.2A.1_2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.2A.1_2.4.3.

6.6.2.2A.1_2.4.2	Test procedure

Same test procedure as in clause 6.6.2.2A.1.4.2 with following exceptions:

-	Instead of Table 6.2.4A.1.5-4 use Table 6.2.4A.1_2.5-4.

6.6.2.2A.1_2.4.3	Message contents

Same message contents as in clause 6.6.2.2A.1.4.3.

6.6.2.2A.1_2.5	Test Requirements

Same test requirements as in clause 6.6.2.2A.1.5 with following exceptions:

-	Instead of Table 6.2.4A.1.5-4 use Table 6.2.4A.1_2.5-4.

##### 6.6.2.2A.2	Additional Spectrum Emission Mask for CA (inter-band DL CA and UL CA)

6.6.2.2A.2.1	Test purpose

To verify that the power of any UE emission shall not exceed specified lever for the specified channel bandwidth for CA under the deployment scenarios where additional requirements are specified.

6.6.2.2A.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 11 and forward that support inter-band DL CA and UL CA but do not support UL 64QAM or UL 256QAM.

6.6.2.2A.2.3	Minimum conformance requirements

The minimum requirements are the same as in section 6.6.2.2.3 per component carrier.

6.6.2.2A.2.4	Test description

6.6.2.2A.2.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state. For Uplink CA Configurations listed in table 6.6.2.2A.2.4.1-1, Additional Spectrum emission requirements are tested.

Table 6.6.2.2A.2.4.1-1: Additional Spectrum Emission mask for inter-band CA

| Uplink CA Configuration | NS-value per CC (in same order as UL CA Configuration column) |  | Applicable spectrum emission requirements (in same order as UL CA Configuration column) |  |
| --- | --- | --- | --- | --- |
| CA_1A-3A CA_1A-5A CA_1A-7A CA_1A-8A CA_1A-18A CA_1A-26A CA_1A-42A | NS_05 | NS_01 | N/A | N/A |
| CA_1A-19A | NS_05 | NS_08 | N/A | N/A |
| CA_1A-21A | NS_05 | NS_09 | N/A | N/A |
| CA_1A-28A | NS_05 | NS_17 | N/A | N/A |
| CA_2A-4A, CA_2A-66A | NS_03 | NS_03 | Table 6.6.2.2.3.1-1 | Table 6.6.2.2.3.1-1 |
| CA_2A-12A, CA_2A-13A, CA_4A-12A, CA_4A-13A, CA_4A-17A | NS_03 | NS_06 | Table 6.6.2.2.3.1-1 | Table 6.6.2.2.3.3-1 |
| CA_2A-30A | NS_03 | NS_21 | Table 6.6.2.2.3.1-1 | Table 6.6.2.2.3.1-1 |
| CA_3A-5A CA_3A-7A CA_3A-8A CA_5A-7A CA_39A-41A CA_41A-42A | NS_01 | NS_01 | N/A | N/A |
| CA_3A-19A | NS_01 | NS_08 | N/A | N/A |
| CA_3A-20A CA_7A-20A | NS_01 | NS_10 | N/A | N/A |
| CA_2A-13A, CA_4A-13A | NS_03 | NS_07 | Table 6.6.2.2.3.1-1 | Table 6.6.2.2.3.3-1 |
| CA_2A-5A CA_2A-7A, CA_4A-5A, CA_4A-7A | NS_03 | NS_01 | Table 6.6.2.2.3.1-1 | N/A |
| CA_5A-66ANote 5 | NS_01 | NS_03 | N/A | Table 6.6.2.2.3.1-1 |
| CA_5A-12A CA_5A-17A | NS_01 | NS_06 | N/A | Table 6.6.2.2.3.3-1 |
| CA_5A-30A | NS_01 | NS_21 | N/A | Table 6.6.2.2.3.1-1 |
| CA_7A-28A CA_18A-28A | NS_01 | NS_17 | N/A | N/A |
| CA_7A-28A | NS_01 | NS_18 | N/A | N/A |
| CA_12A-30A | NS_06 | NS_21 | Table 6.6.2.2.3.3-1 | Table 6.6.2.2.3.3-1 |
| CA_12A-66ANote 5 | NS_06 | NS_03 | Table 6.6.2.2.3.3-1 | Table 6.6.2.2.3.1-1 |
| CA_19A-21A | NS_08 | NS_09 | N/A | N/A |
| CA_30A-48A | NS_21 | NS_27 | Table 6.6.2.2.3.1-1 | Table 6.6.2.2.3.4-1 |
| Note 1: For CCs with NS_01, there are no additional requirements defined in 36.101. Only CCs with NS-value different than NS_01 needs to be tested. Note 2: A-MPR values for NS-values in this table are the same as in section 6.2.4.3. Note 3: Selection of CA configurations and NS-values to test is derived in TR 36.905 [19]. Note 4: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column. Note 5: As an exception to note 4, PCC is mapped to the second band of the combination as listed in Uplink CA Configuration column. |  |  |  |  |

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in Table 5.4.2A.1-2 for CA configurations with two bands or Table 5.4.2A.1-2a for CA configurations with three bands. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in tables 6.6.2.2A.2.4.1-2 to 6.6.2.2A.2.4.1-9. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.2A.2.4.1-2: Test Configuration Table (network signalled value NS_01/NS_03 or network signalled value NS_03/NS_01)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and high range for PCC Mid range for SCC (Note 3) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg(Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | NS_03 CC | NS_01 CC | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 6 | 8 |
| 2 | 25 | 25 |  | 16QAM | 16QAM | 25 | 25 |
| 3 | 25 | 25 |  | 16QAM | QPSK | 25 | 8 |
| 4 | 50 | 50 |  | QPSK | QPSK | 6 | 12 |
| 5 | 50 | 50 |  | 16QAM | 16QAM | 50 | 50 |
| 6 | 50 | 50 |  | 16QAM | QPSK | 50 | 12 |
| 7 | 50 | 100 |  | QPSK | QPSK | 6 | 18 |
| 8 | 50 | 100 |  | 16QAM | 16QAM | 50 | 100 |
| 9 | 50 | 100 |  | 16QAM | QPSK | 50 | 18 |
| 10 | 100 | 50 |  | QPSK | QPSK | 10 | 12 |
| 11 | 100 | 50 |  | 16QAM | 16QAM | 100 | 50 |
| 12 | 100 | 50 |  | 16QAM | QPSK | 100 | 12 |
| 13 | 100 | 100 |  | QPSK | QPSK | 10 | 18 |
| 14 | 100 | 100 |  | 16QAM | 16QAM | 100 | 100 |
| 15 | 100 | 100 |  | 16QAM | QPSK | 100 | 18 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg. Only the combination with the highest NRB_PCC is tested. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBmax-LCRB+1. For Mid range place test point with RBstart=0 and RBstart=RBmax-LCRB+1. Note 4: Mapping of NS-value per CC and PCC/SCC selection are specified in Table 6.6.2.2A.2.4.1-1. |  |  |  |  |  |  |  |

Table 6.6.2.2A.2.4.1-3: Test Configuration Table (network signalled value NS_01/NS_06)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC (Note 3) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg(Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_03) | SCC (NS_07) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 6 | 6 |
| 2 | 25 | 25 |  | 16QAM | 16QAM | 25 | 9 |
| 3 | 25 | 25 |  | QPSK | 16QAM | 6 | 9 |
| 4 | 50 | 50 |  | QPSK | QPSK | 12 | 12 |
| 5 | 50 | 50 |  | 16QAM | 16QAM | 50 | 15 |
| 6 | 50 | 50 |  | QPSK | 16QAM | 12 | 15 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg. Only the combination with the highest NRB_PCC is tested. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBmax-LCRB+1. |  |  |  |  |  |  |  |

Table 6.6.2.2A.2.4.1-4: Test Configuration Table (network signalled value NS_01/NS_21)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_01) | SCC (NS_21) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 8 | 8 |
| 2 | 25 | 25 |  | 16QAM | 16QAM | 25 | 25 |
| 3 | 25 | 25 |  | QPSK | 16QAM | 8 | 25 |
| 4 (Note 4) | 50 | 50 |  | QPSK | QPSK | 12 | 12@7 |
| 5 (Note 5) | 50 | 50 |  | QPSK | QPSK | 12 | 12@31 |
| 6 | 50 | 50 |  | 16QAM | 16QAM | 50 | 1 |
| 7 | 50 | 50 |  | QPSK | 16QAM | 12 | 1 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column in Table 6.6.2.2A.2.4.1-1. Note 3: For partial RB allocation unless RBstart is explicitly defined, for Low range place test point with RBstart=0, for High range place test point with RBstart=RBmax-LCRB+1. Note 4: Applicable only to Low range frequency testing. Note 5: Applicable only to High range frequency testing. |  |  |  |  |  |  |  |

Table 6.6.2.2A.2.4.1-5: Test Configuration Table (network signalled value NS_03/NS_03)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC (Note 3) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg (Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_03) | SCC (NS_03) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 15 | 25 | N/A for this test | QPSK | QPSK | 4 | 6 |
| 2 | 15 | 25 |  | 16QAM | 16QAM | 15 | 25 |
| 3 | 15 | 25 |  | 16QAM | QPSK | 15 | 6 |
| 4 | 25 | 25 |  | QPSK | QPSK | 6 | 6 |
| 5 | 25 | 25 |  | 16QAM | 16QAM | 25 | 25 |
| 6 | 25 | 25 |  | 16QAM | QPSK | 25 | 6 |
| 7 | 50 | 50 |  | QPSK | QPSK | 6 | 6 |
| 8 | 50 | 50 |  | 16QAM | 16QAM | 50 | 50 |
| 9 | 50 | 50 |  | 16QAM | QPSK | 50 | 6 |
| 10 | 100 | 100 |  | QPSK | QPSK | 10 | 10 |
| 11 | 100 | 100 |  | 16QAM | 16QAM | 100 | 100 |
| 12 | 100 | 100 |  | 16QAM | QPSK | 100 | 10 |
| 13 | 6 | 25 |  | QPSK | QPSK | 5 | 6 |
| 14 | 6 | 25 |  | 16QAM | 16QAM | 6 | 25 |
| 15 | 6 | 25 |  | 16QAM | QPSK | 6 | 6 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg. Only the combination with the highest NRB_PCC is tested. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBmax-LCRB+1. |  |  |  |  |  |  |  |

Table 6.6.2.2A.2.4.1-6: Test Configuration Table (network signalled value NS_03/NS_06)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC (Note 3) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg(Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_03) | SCC (NS_06) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 15 | 25 | N/A for this test | QPSK | QPSK | 4 | 8 |
| 2 | 15 | 25 |  | 16QAM | 16QAM | 15 | 25 |
| 3 | 15 | 25 |  | 16QAM | QPSK | 15 | 8 |
| 4 | 25 | 15 |  | QPSK | QPSK | 6 | 4 |
| 5 | 25 | 15 |  | 16QAM | 16QAM | 25 | 15 |
| 6 | 25 | 15 |  | 16QAM | QPSK | 25 | 4 |
| 7 | 25 | 25 |  | QPSK | QPSK | 6 | 8 |
| 8 | 25 | 25 |  | 16QAM | 16QAM | 25 | 25 |
| 9 | 25 | 25 |  | 16QAM | QPSK | 25 | 8 |
| 10 | 25 | 50 |  | QPSK | QPSK | 6 | 12 |
| 11 | 25 | 50 |  | 16QAM | 16QAM | 25 | 50 |
| 12 | 25 | 50 |  | 16QAM | QPSK | 25 | 12 |
| 13 | 50 | 50 |  | QPSK | QPSK | 6 | 12 |
| 14 | 50 | 50 |  | 16QAM | 16QAM | 50 | 50 |
| 15 | 50 | 50 |  | 16QAM | QPSK | 50 | 12 |
| 16 | 100 | 50 |  | QPSK | QPSK | 10 | 12 |
| 17 | 100 | 50 |  | 16QAM | 16QAM | 100 | 50 |
| 18 | 100 | 50 |  | 16QAM | QPSK | 100 | 12 |
| 19 | 75 | 25 |  | QPSK | QPSK | 8 | 8 |
| 20 | 75 | 25 |  | 16QAM | 16QAM | 75 | 25 |
| 21 | 75 | 25 |  | 16QAM | QPSK | 75 | 8 |
| 22 | 6 | 25 |  | QPSK | QPSK | 5 | 8 |
| 23 | 6 | 25 |  | 16QAM | 16QAM | 6 | 25 |
| 24 | 6 | 25 |  | 16QAM | QPSK | 6 | 8 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg. Only the combination with the highest NRB_PCC is tested. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBmax-LCRB+1. |  |  |  |  |  |  |  |

Table 6.6.2.2A.2.4.1-7: Test Configuration Table (network signalled value NS_03/NS_07)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC (Note 3) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg(Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_03) | SCC (NS_07) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 50 | N/A for this test | QPSK | QPSK | 6 | 6@13 |
| 2 | 25 | 50 |  | 16QAM | 16QAM | 25 | 50 |
| 3 | 25 | 50 |  | QPSK | 16QAM | 6 | 50 |
| 4 | 50 | 50 |  | QPSK | QPSK | 6 | 6@13 |
| 5 | 50 | 50 |  | 16QAM | 16QAM | 50 | 50 |
| 6 | 50 | 50 |  | QPSK | 16QAM | 6 | 50 |
| 7 | 100 | 50 |  | QPSK | QPSK | 10 | 6@13 |
| 8 | 100 | 50 |  | 16QAM | 16QAM | 100 | 50 |
| 9 | 100 | 50 |  | QPSK | 16QAM | 10 | 50 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg. Only the combination with the highest NRB_PCC is tested. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBmax-LCRB+1. |  |  |  |  |  |  |  |

Table 6.6.2.2A.2.4.1-8: Test Configuration Table (network signalled value NS_03/NS_21)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_03) | SCC (NS_21) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 6 | 8 |
| 2 | 25 | 25 |  | 16QAM | 16QAM | 25 | 25 |
| 3 | 25 | 25 |  | 16QAM | QPSK | 25 | 8 |
| 4 (Note 4) | 100 | 50 |  | QPSK | QPSK | 10 | 12@7 |
| 5 (Note 5) | 100 | 50 |  | QPSK | QPSK | 10 | 12@31 |
| 6 | 100 | 50 |  | 16QAM | 16QAM | 100 | 1 |
| 7 | 100 | 50 |  | QPSK | 16QAM | 10 | 1 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column in Table 6.6.2.2A.2.4.1-1. Note 3: For partial RB allocation unless RBstart is explicitly defined, for Low range place test point with RBstart=0, for High range place test point with RBstart=RBmax-LCRB+1. Note 4: Applicable only to Low range frequency testing. Note 5: Applicable only to High range frequency testing. |  |  |  |  |  |  |  |

Table 6.6.2.2A.2.4.1-9: Test Configuration Table (network signalled value NS_06/NS_21)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_06) | SCC (NS_21) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 8 | 8 |
| 2 | 25 | 25 |  | 16QAM | 16QAM | 25 | 25 |
| 3 | 25 | 25 |  | 16QAM | QPSK | 25 | 8 |
| 4 (Note 4) | 50 | 50 |  | QPSK | QPSK | 12 | 12@7 |
| 5 (Note 5) | 50 | 50 |  | QPSK | QPSK | 12 | 12@31 |
| 6 | 50 | 50 |  | 16QAM | 16QAM | 50 | 1 |
| 7 | 50 | 50 |  | QPSK | 16QAM | 12 | 1 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column in Table 6.6.2.2A.2.4.1-1. Note 3: For partial RB allocation unless RBstart is explicitly defined, for Low range place test point with RBstart=0, for High range place test point with RBstart=RBmax-LCRB+1. Note 4: Applicable only to Low range frequency testing. Note 5: Applicable only to High range frequency testing. |  |  |  |  |  |  |  |

Table 6.6.2.2A.2.4.1-10: Test Configuration Table (network signalled value NS_21/NS_27)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_21) | SCC (NS_27) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 8 | 8 |
| 2 | 25 | 25 |  | 16QAM | 16QAM | 25 | 25 |
| 3 | 25 | 25 |  | 16QAM | QPSK | 25 | 8 |
| 4 (Note 4) | 50 | 100 |  | QPSK | QPSK | 12@7 | 20 |
| 5 (Note 5) | 50 | 100 |  | QPSK | QPSK | 12@31 | 20 |
| 6 | 50 | 100 |  | 16QAM | 16QAM | 50 | 20 |
| 7 | 50 | 100 |  | QPSK | 16QAM | 12 | 20 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column in Table 6.2.4A.2.4.1-1. Note 3: For partial RB allocation unless RBstart is explicitly defined, for Low range place test point with RBstart=0, for High range place test point with RBstart=RBend-LCRB+1. Note 4: Applicable only to Low range frequency testing. Note 5: Applicable only to High range frequency testing. |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] clause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.2A.2.4.1-2 to 6.6.2.2A.2.4.1-9.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.2A.2.4.3.

6.6.2.2A.2.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.6.2.2A.2.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.6.2.2A.2.4.1-2 to 6.6.2.2A.2.4.1-9 on both PCC and SCC. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

6.	Measure the mean power over all component carriers in the CA configuration of the radio access mode, which shall meet the requirements described in Tables 6.2.4A.2.5-2 to 6.2.4A.2.5-12. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

7.	Measure the power of the transmitted signal for PCC if NS-value other than NS_01 with a measurement filter of bandwidths according to tables 6.6.2.2A.2.5.1-1 to 6.6.2.2A.2.5.1-4 as applicable and using a rms detector. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

8.	Measure the power of the transmitted signal for SCC if NS-value other than NS_01 with a measurement filter of bandwidths according to tables 6.6.2.2A.2.5.1-1 to 6.6.2.2A.2.5.1-4 as applicable and using a rms detector. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.6.2.2A.2.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6. The following exceptions apply for each network signalled value. These exceptions indicate that the UE shall meet the additional spectrum emission requirement for a specific deployment scenario.

6.6.2.2A.2.4.3.1	Message contents exceptions (network signalled value " NS_03" on PCC)

1.	Information element additionalSpectrumEmission is set to NS_03 for PCC. This can be set in the SystemInformationblockType2 as part of the cell broadcast message.

Table 6.6.2.2A.2.4.3.1-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_03" on PCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 3 (NS_03) |  |  |

6.6.2.2A.2.4.3.2	Message contents exceptions (network signalled value " NS_03" on SCC)

1.	Information element additionalSpectrumEmissionSCell-r10 is set to NS_03 for SCC. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages.

Table 6.6.2.2A.2.4.3.2-1: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "NS_03" on SCC

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 3 (NS_03) |  |  |

6.6.2.2A.2.4.3.3	Message contents exceptions (network signalled value " NS_06" on SCC)

1.	Information element additionalSpectrumEmissionSCell-r10 is set to NS_06 for SCC. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages.

Table 6.6.2.2A.2.4.3.3-1: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "NS_06" on SCC

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 6 (NS_06) |  |  |

6.6.2.2A.2.4.3.4	Message contents exceptions (network signalled value " NS_07" on SCC)

1.	Information element additionalSpectrumEmissionSCell-r10 is set to NS_07 for SCC. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages.

Table 6.6.2.2A.1.4.3.4-1: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "NS_07" on SCC

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 7 (NS_07) |  |  |

6.6.2.2A.2.4.3.5	Message contents exceptions (network signalled value " NS_21" on SCC)

1.	Information element additionalSpectrumEmission is set to NS_21 for SCC. This can be set in the SystemInformationblockType2 as part of the cell broadcast message.

Table 6.6.2.2A.2.4.3.5-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_21" on SCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 21 (NS_21) |  |  |

2.	Information element additionalSpectrumEmissionSCell-r10 is set to NS_21 for SCC. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages.

Table 6.6.2.2A.2.4.3.5-2: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "NS_21" on SCC

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 21 (NS_21) |  |  |

6.6.2.2A.2.4.3.6	Message contents exceptions (network signalled value " NS_21" on PCC)

1.	Information element additionalSpectrumEmission is set to NS_21 for PCC. This can be set in the SystemInformationblockType2 as part of the cell broadcast message.

Table 6.6.2.2A.2.4.3.6-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_21" on PCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 21 (NS_21) |  |  |

6.6.2.2A.2.4.3.7	Message contents exceptions (network signalled value " NS_27" on SCC)

1.	Information element additionalSpectrumEmission is set to NS_27 for SCC. This can be set in the SystemInformationblockType2 as part of the cell broadcast message.

Table 6.6.2.2A.2.4.3.7-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_27" on SCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 27 (NS_27) |  |  |

2.	Information element additionalSpectrumEmissionSCell-r10 is set to NS_27 for SCC. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages.

Table 6.6.2.2A.2.4.3.7-2: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "NS_27" on SCC

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 27 (NS_27) |  |  |

6.6.2.2A.2.5	Test Requirements

The measured UE mean power with the aggregated channel bandwidth as specified in clause 5.4.2A, derived in step 6 , shall fulfil requirements in Tables 6.2.4A.2.5-1 to 6.2.4A.2.5-12 as appropriate,

and

The power of any UE emission per component carrier derived in step 7 and 8 shall not exceed the levels specified in Tables 6.6.2.2A.2.5.1-1 to 6.6.2.2A.2.5.1-4.

Table 6.6.2.2A.2.5-1: Additional requirements (network signalled value "NS_03", “NS_11”, “NS_20” and “NS_21”), E-UTRA bands ≤ 3GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.5 | -11.5 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 1-2.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 |  |  |  |  | 1 MHz |
| 6-10 |  |  | -23.5 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.5 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.5 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_03 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

Table 6.6.2.2A.2.5-2: Additional requirements (network signalled value "NS_03", “NS_11”, “NS_20” and “NS_21”), 3GHz < E-UTRA bands ≤ 4.2GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.2 | -11.2 | -13.2 | -16.2 | -18.2 | -19.2 | 30 kHz |
| 1-2.5 | -11.2 | -11.2 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 2.5-2.8 | -23.2 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.2 |  |  |  |  | 1 MHz |
| 6-10 |  |  | -23.2 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.2 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.2 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.2 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_03 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

Table 6.6.2.2A.2.5-3: Additional requirements (network signalled value "NS_06" or "NS_07"), E-UTRA bands ≤ 3GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | Measurement bandwidth |
| 0-0.1 | -11.5 | -11.5 | -13.5 | -16.5 | 30 kHz |
| 0.1-1 | -11.5 | -11.5 | -11.5 | -11.5 | 100 kHz |
| 1-2.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 |  |  | 1 MHz |
| 6-10 |  |  | -23.5 |  | 1 MHz |
| 10-15 |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.085 MHz. The first and last measurement position with a 100 kHz filter is at ΔfOOB equals to 0.15 MHz and 0.95 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_06 and NS_07 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |

Table 6.6.2.2A.2.5-4: Additional requirements (network signalled value "NS_06" or "NS_07"), 3GHz < E-UTRA bands ≤ 4.2GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | Measurement bandwidth |
| 0-0.1 | -11.2 | -11.2 | -13.2 | -16.2 | 30 kHz |
| 0.1-1 | -11.2 | -11.2 | -11.2 | -11.2 | 100 kHz |
| 1-2.5 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 2.5-2.8 | -23.2 |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  | 1 MHz |
| 5-6 |  | -23.2 |  |  | 1 MHz |
| 6-10 |  |  | -23.2 |  | 1 MHz |
| 10-15 |  |  |  | -23.2 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.085 MHz. The first and last measurement position with a 100 kHz filter is at ΔfOOB equals to 0.15 MHz and 0.95 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel. Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_06 and NS_07 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |

Table 6.6.2.2A.2.5-5: Additional requirements (network signalled value “NS_27” or “NS_43”), 3GHz < E-UTRA bands ≤ 4.2GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -11.2 | -11.2 | -11.2 | -11.2 | Note 6 |
| 1-10 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 10-15 |  | -23.2 | -23.2 | -23.2 | 1 MHz |
| 15-20 |  |  | -23.2 | -23.2 | 1 MHz |
| 20-25 |  |  |  | -23.2 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.085 MHz. The first and last measurement position with a 100 kHz filter is at ΔfOOB equals to 0.15 MHz and 0.95 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_06 and NS_07 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. Note 6: The measurement bandwidth is 1% of the applicable E-UTRA channel bandwidth. |  |  |  |  |  |

NOTE:	(only for emission measurement) As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

##### 6.6.2.2A.2_1	Additional Spectrum Emission Mask for CA (inter-band DL CA and UL CA) for UL 64QAM

Editor's note: For a transition period until RAN5#83, this test case in version 15.3.1 of 36.521-1 shall be used. This is to ensure no test coverage is lost before the UL 256QAM test case becomes available.

6.6.2.2A.2_1.1	Test purpose

To verify that the power of any UE emission shall not exceed specified lever for the specified channel bandwidth for CA under the deployment scenarios where additional requirements are specified.

6.6.2.2A.2_1.2	Test applicability

This test case applies to all types of E-UTRA UE release 13 and forward that support inter-band DL CA and UL CA and UL 64QAM but do not support UL 256QAM.

6.6.2.2A.2_1.3	Minimum conformance requirements

The minimum requirements are the same as in section 6.6.2.2A.2.3.

6.6.2.2A.2_1.4	Test description

6.6.2.2A.2_1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state. For Uplink CA Configurations listed in table 6.6.2.2A.2_1.4.1-1, Additional Spectrum emission requirements are tested.

Table 6.6.2.2A.2_1.4.1-1: Additional Spectrum Emission mask for inter-band CA

| Uplink CA Configuration | NS-value per CC (in same order as UL CA Configuration column) |  | Applicable spectrum emission requirements (in same order as UL CA Configuration column) |  |
| --- | --- | --- | --- | --- |
| CA_2A-4A, CA_2A-66A | NS_03 | NS_03 | Table 6.6.2.2.3.1-1 | Table 6.6.2.2.3.1-1 |
| CA_2A-12A, CA_2A-13A, CA_4A-12A, CA_4A-13A, CA_4A-17A | NS_03 | NS_06 | Table 6.6.2.2.3.1-1 | Table 6.6.2.2.3.3-1 |
| CA_2A-13A, CA_4A-13A | NS_03 | NS_07 | Table 6.6.2.2.3.1-1 | Table 6.6.2.2.3.3-1 |
| CA_2A-5A, CA_2A-7A, CA_4A-5A, CA_4A-7A | NS_03 | NS_01 | Table 6.6.2.2.3.1-1 | N/A |
| CA_5A-66ANote5 | NS_01 | NS_03 | N/A | Table 6.6.2.2.3.1-1 |
| CA_2A-30A | NS_03 | NS_21 | Table 6.6.2.2.3.1-1 | Table 6.6.2.2.3.1-1 |
| CA_5A-12A | NS_01 | NS_06 | N/A | Table 6.6.2.2.3.3-1 |
| CA_5A-30A | NS_01 | NS_21 | N/A | Table 6.6.2.2.3.1-1 |
| CA_12A-30A | NS_06 | NS_21 | Table 6.6.2.2.3.3-1 | Table 6.6.2.2.3.3-1 |
| CA_1A-3A | NS_05 | NS_01 | N/A | N/A |
| CA_1A-19A | NS_05 | NS_08 | N/A | N/A |
| CA_1A-21A | NS_05 | NS_09 | N/A | N/A |
| CA_3A-19A | NS_01 | NS_08 | N/A | N/A |
| CA_19A-21A | NS_08 | NS_09 | N/A | N/A |
| Note 1: For CCs with NS_01, there are no additional requirements defined in 36.101. Only CCs with NS-value different than NS_01 needs to be tested. Note 2: A-MPR values for NS-values in this table are the same as in section 6.2.4.3. Note 3: Selection of CA configurations and NS-values to test is derived in TR 36.905 [19]. Note 4: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column. Note 5: As an exception to note 4, PCC is mapped to the second band of the combination as listed in Uplink CA Configuration column. |  |  |  |  |

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-2. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in tables 6.6.2.2A.2_1.4.1-2 to 6.6.2.2A.2_1.4.1-9. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.2A.2_1.4.1-2: Test Configuration Table (network signalled value NS_01/NS_03 or network signalled value NS_03/NS_01)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and high range for PCC Mid range for SCC (Note 3) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg(Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | NS_03 CC | NS_01 CC | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 6 | 8 |
| 2 | 25 | 25 |  | 64QAM | 64QAM | 25 | 25 |
| 3 | 25 | 25 |  | 64QAM | QPSK | 25 | 8 |
| 4 | 50 | 50 |  | QPSK | QPSK | 6 | 12 |
| 5 | 50 | 50 |  | 64QAM | 64QAM | 50 | 50 |
| 6 | 50 | 50 |  | 64QAM | QPSK | 50 | 12 |
| 7 | 50 | 100 |  | QPSK | QPSK | 6 | 18 |
| 8 | 50 | 100 |  | 64QAM | 64QAM | 50 | 100 |
| 9 | 50 | 100 |  | 64QAM | QPSK | 50 | 18 |
| 10 | 100 | 50 |  | QPSK | QPSK | 10 | 12 |
| 11 | 100 | 50 |  | 64QAM | 64QAM | 100 | 50 |
| 12 | 100 | 50 |  | 64QAM | QPSK | 100 | 12 |
| 13 | 100 | 100 |  | QPSK | QPSK | 10 | 18 |
| 14 | 100 | 100 |  | 64QAM | 64QAM | 100 | 100 |
| 15 | 100 | 100 |  | 64QAM | QPSK | 100 | 18 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg. Only the combination with the highest NRB_PCC is tested. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBmax-LCRB+1. For Mid range place test point with RBstart=0 and RBstart=RBmax-LCRB+1. Note 4: Mapping of NS-value per CC and PCC/SCC selection are specified in Table 6.6.2.2A.2_1.4.1-1. |  |  |  |  |  |  |  |

Table 6.6.2.2A.2_1.4.1-3: Test Configuration Table (network signalled value NS_01/NS_06)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC (Note 3) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg(Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_03) | SCC (NS_07) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 6 | 6 |
| 2 | 25 | 25 |  | 64QAM | 64QAM | 25 | 9 |
| 3 | 25 | 25 |  | QPSK | 64QAM | 6 | 9 |
| 4 | 50 | 50 |  | QPSK | QPSK | 12 | 12 |
| 5 | 50 | 50 |  | 64QAM | 64QAM | 50 | 15 |
| 6 | 50 | 50 |  | QPSK | 64QAM | 12 | 15 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg. Only the combination with the highest NRB_PCC is tested. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBmax-LCRB+1. |  |  |  |  |  |  |  |

Table 6.6.2.2A.2_1.4.1-4: Test Configuration Table (network signalled value NS_01/NS_21)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_01) | SCC (NS_21) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 8 | 8 |
| 2 | 25 | 25 |  | 64QAM | 64QAM | 25 | 25 |
| 3 | 25 | 25 |  | QPSK | 64QAM | 8 | 25 |
| 4 (Note 4) | 50 | 50 |  | QPSK | QPSK | 12 | 12@7 |
| 5 (Note 5) | 50 | 50 |  | QPSK | QPSK | 12 | 12@31 |
| 6 | 50 | 50 |  | 64QAM | 64QAM | 50 | 1 |
| 7 | 50 | 50 |  | QPSK | 64QAM | 12 | 1 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column in Table 6.6.2.2A.2_1.4.1-1. Note 3: For partial RB allocation unless RBstart is explicitly defined, for Low range place test point with RBstart=0, for High range place test point with RBstart=RBmax-LCRB+1. Note 4: Applicable only to Low range frequency testing. Note 5: Applicable only to High range frequency testing. |  |  |  |  |  |  |  |

Table 6.6.2.2A.2_1.4.1-5: Test Configuration Table (network signalled value NS_03/NS_03)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC (Note 3) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg (Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_03) | SCC (NS_03) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 15 | 25 | N/A for this test | QPSK | QPSK | 4 | 6 |
| 2 | 15 | 25 |  | 64QAM | 64QAM | 15 | 25 |
| 3 | 15 | 25 |  | 64QAM | QPSK | 15 | 6 |
| 4 | 25 | 25 |  | QPSK | QPSK | 6 | 6 |
| 5 | 25 | 25 |  | 64QAM | 64QAM | 25 | 25 |
| 6 | 25 | 25 |  | 64QAM | QPSK | 25 | 6 |
| 7 | 50 | 50 |  | QPSK | QPSK | 6 | 6 |
| 8 | 50 | 50 |  | 64QAM | 64QAM | 50 | 50 |
| 9 | 50 | 50 |  | 64QAM | QPSK | 50 | 6 |
| 10 | 100 | 100 |  | QPSK | QPSK | 10 | 10 |
| 11 | 100 | 100 |  | 64QAM | 64QAM | 100 | 100 |
| 12 | 100 | 100 |  | 64QAM | QPSK | 100 | 10 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg. Only the combination with the highest NRB_PCC is tested. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBmax-LCRB+1. |  |  |  |  |  |  |  |

Table 6.6.2.2A.2_1.4.1-6: Test Configuration Table (network signalled value NS_03/NS_06)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC (Note 3) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg(Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_03) | SCC (NS_06) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 15 | 25 | N/A for this test | QPSK | QPSK | 4 | 8 |
| 2 | 15 | 25 |  | 64QAM | 64QAM | 15 | 25 |
| 3 | 15 | 25 |  | 64QAM | QPSK | 15 | 8 |
| 4 | 25 | 15 |  | QPSK | QPSK | 6 | 4 |
| 5 | 25 | 15 |  | 64QAM | 64QAM | 25 | 15 |
| 6 | 25 | 15 |  | 64QAM | QPSK | 25 | 4 |
| 7 | 25 | 25 |  | QPSK | QPSK | 6 | 8 |
| 8 | 25 | 25 |  | 64QAM | 64QAM | 25 | 25 |
| 9 | 25 | 25 |  | 64QAM | QPSK | 25 | 8 |
| 10 | 25 | 50 |  | QPSK | QPSK | 6 | 12 |
| 11 | 25 | 50 |  | 64QAM | 64QAM | 25 | 50 |
| 12 | 25 | 50 |  | 64QAM | QPSK | 25 | 12 |
| 13 | 50 | 50 |  | QPSK | QPSK | 6 | 12 |
| 14 | 50 | 50 |  | 64QAM | 64QAM | 50 | 50 |
| 15 | 50 | 50 |  | 64QAM | QPSK | 50 | 12 |
| 16 | 100 | 50 |  | QPSK | QPSK | 10 | 12 |
| 17 | 100 | 50 |  | 64QAM | 64QAM | 100 | 50 |
| 18 | 100 | 50 |  | 64QAM | QPSK | 100 | 12 |
| 19 | 75 | 25 |  | QPSK | QPSK | 8 | 8 |
| 20 | 75 | 25 |  | 64QAM | 64QAM | 75 | 25 |
| 21 | 75 | 25 |  | 64QAM | QPSK | 75 | 8 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg. Only the combination with the highest NRB_PCC is tested. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBmax-LCRB+1. |  |  |  |  |  |  |  |

Table 6.6.2.2A.2_1.4.1-7: Test Configuration Table (network signalled value NS_03/NS_07)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC (Note 3) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg(Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_03) | SCC (NS_07) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 50 | N/A for this test | QPSK | QPSK | 6 | 6@13 |
| 2 | 25 | 50 |  | 64QAM | 64QAM | 25 | 50 |
| 3 | 25 | 50 |  | QPSK | 64QAM | 6 | 50 |
| 4 | 50 | 50 |  | QPSK | QPSK | 6 | 6@13 |
| 5 | 50 | 50 |  | 64QAM | 64QAM | 50 | 50 |
| 6 | 50 | 50 |  | QPSK | 64QAM | 6 | 50 |
| 7 | 100 | 50 |  | QPSK | QPSK | 10 | 6@13 |
| 8 | 100 | 50 |  | 64QAM | 64QAM | 100 | 50 |
| 9 | 100 | 50 |  | QPSK | 64QAM | 10 | 50 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg. Only the combination with the highest NRB_PCC is tested. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBmax-LCRB+1. |  |  |  |  |  |  |  |

Table 6.6.2.2A.2_1.4.1-8: Test Configuration Table (network signalled value NS_03/NS_21)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_03) | SCC (NS_21) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 6 | 8 |
| 2 | 25 | 25 |  | 64QAM | 64QAM | 25 | 25 |
| 3 | 25 | 25 |  | 64QAM | QPSK | 25 | 8 |
| 4 (Note 4) | 100 | 50 |  | QPSK | QPSK | 10 | 12@7 |
| 5 (Note 5) | 100 | 50 |  | QPSK | QPSK | 10 | 12@31 |
| 6 | 100 | 50 |  | 64QAM | 64QAM | 100 | 1 |
| 7 | 100 | 50 |  | QPSK | 64QAM | 10 | 1 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column in Table 6.6.2.2A.2_1.4.1-1. Note 3: For partial RB allocation unless RBstart is explicitly defined, for Low range place test point with RBstart=0, for High range place test point with RBstart=RBmax-LCRB+1. Note 4: Applicable only to Low range frequency testing. Note 5: Applicable only to High range frequency testing. |  |  |  |  |  |  |  |

Table 6.6.2.2A.2_1.4.1-9: Test Configuration Table (network signalled value NS_06/NS_21)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_06) | SCC (NS_21) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 8 | 8 |
| 2 | 25 | 25 |  | 64QAM | 64QAM | 25 | 25 |
| 3 | 25 | 25 |  | 64QAM | QPSK | 25 | 8 |
| 4 (Note 4) | 50 | 50 |  | QPSK | QPSK | 12 | 12@7 |
| 5 (Note 5) | 50 | 50 |  | QPSK | QPSK | 12 | 12@31 |
| 6 | 50 | 50 |  | 64QAM | 64QAM | 50 | 1 |
| 7 | 50 | 50 |  | QPSK | 64QAM | 12 | 1 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column in Table 6.6.2.2A.2_1.4.1-1. Note 3: For partial RB allocation unless RBstart is explicitly defined, for Low range place test point with RBstart=0, for High range place test point with RBstart=RBmax-LCRB+1. Note 4: Applicable only to Low range frequency testing. Note 5: Applicable only to High range frequency testing. |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] clause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.2A.2_1.4.1-2 to 6.6.2.2A.2_1.4.1-9.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.2A.2_1.4.3.

6.6.2.2A.2_1.4.2	Test procedure

Same test procedure as in clause 6.6.2.2A.2.4.2.

6.6.2.2A.2_1.4.3	Message contents

Same message contents as in clause 6.6.2.2A.2.4.3.

6.6.2.2A.2_1.5	Test Requirements

Same test requirements as in clause 6.6.2.2A.2.5.

##### 6.6.2.2A.2_2	Additional Spectrum Emission Mask for CA (inter-band DL CA and UL CA) for UL 256QAM

6.6.2.2A.2_2.1	Test purpose

To verify that the power of any UE emission shall not exceed specified lever for the specified channel bandwidth for CA under the deployment scenarios where additional requirements are specified.

6.6.2.2A.2_2.2	Test applicability

This test case applies to all types of E-UTRA UE release 14 and forward that support inter-band DL CA and UL CA and UL 256QAM.

6.6.2.2A.2_2.3	Minimum conformance requirements

The minimum requirements are the same as in section 6.6.2.2A.2.3.

6.6.2.2A.2_2.4	Test description

6.6.2.2A.2_2.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state. For Uplink CA Configurations listed in table 6.6.2.2A.2_2.4.1-1, Additional Spectrum emission requirements are tested.

Table 6.6.2.2A.2_2.4.1-1 Additional Spectrum Emission mask for inter-band CA

| Uplink CA Configuration | NS-value per CC (in same order as UL CA Configuration column) |  | Applicable spectrum emission requirements (in same order as UL CA Configuration column) |  |
| --- | --- | --- | --- | --- |
| CA_2A-4A, CA_2A-66A | NS_03 | NS_03 | Table 6.6.2.2.3.1-1 | Table 6.6.2.2.3.1-1 |
| CA_2A-12A, CA_2A-13A, CA_4A-12A, CA_4A-13A, CA_4A-17A | NS_03 | NS_06 | Table 6.6.2.2.3.1-1 | Table 6.6.2.2.3.3-1 |
| CA_2A-13A, CA_4A-13A | NS_03 | NS_07 | Table 6.6.2.2.3.1-1 | Table 6.6.2.2.3.3-1 |
| CA_2A-5A, CA_2A-7A, CA_4A-5A, CA_4A-7A | NS_03 | NS_01 | Table 6.6.2.2.3.1-1 | N/A |
| CA_5A-66ANote5 | NS_01 | NS_03 | N/A | Table 6.6.2.2.3.1-1 |
| CA_2A-30A | NS_03 | NS_21 | Table 6.6.2.2.3.1-1 | Table 6.6.2.2.3.1-1 |
| CA_5A-12A | NS_01 | NS_06 | N/A | Table 6.6.2.2.3.3-1 |
| CA_5A-30A | NS_01 | NS_21 | N/A | Table 6.6.2.2.3.1-1 |
| CA_12A-30A | NS_06 | NS_21 | Table 6.6.2.2.3.3-1 | Table 6.6.2.2.3.3-1 |
| CA_1A-3A | NS_05 | NS_01 | N/A | N/A |
| CA_1A-19A | NS_05 | NS_08 | N/A | N/A |
| CA_1A-21A | NS_05 | NS_09 | N/A | N/A |
| CA_3A-19A | NS_01 | NS_08 | N/A | N/A |
| CA_19A-21A | NS_08 | NS_09 | N/A | N/A |
| Note 1: For CCs with NS_01, there are no additional requirements defined in 36.101. Only CCs with NS-value different than NS_01 needs to be tested. Note 2: A-MPR values for NS-values in this table are the same as in section 6.2.4.3. Note 3: Selection of CA configurations and NS-values to test is derived in TR 36.905 [19]. Note 4: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column. Note 5: As an exception to note 4, PCC is mapped to the second band of the combination as listed in Uplink CA Configuration column. |  |  |  |  |

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-2. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in tables 6.6.2.2A.2_2.4.1-2 to 6.6.2.2A.2.4.1-9. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.2A.2_2.4.1-2: Test Configuration Table (network signalled value NS_01/NS_03 or network signalled value NS_03/NS_01)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and high range for PCC Mid range for SCC (Note 3) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg(Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | NS_03 CC | NS_01 CC | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 6 | 8 |
| 2 | 25 | 25 |  | 256QAM | 256QAM | 25 | 25 |
| 3 | 25 | 25 |  | 256QAM | QPSK | 25 | 8 |
| 4 | 50 | 50 |  | QPSK | QPSK | 6 | 12 |
| 5 | 50 | 50 |  | 256QAM | 256QAM | 50 | 50 |
| 6 | 50 | 50 |  | 256QAM | QPSK | 50 | 12 |
| 7 | 50 | 100 |  | QPSK | QPSK | 6 | 18 |
| 8 | 50 | 100 |  | 256QAM | 256QAM | 50 | 100 |
| 9 | 50 | 100 |  | 256QAM | QPSK | 50 | 18 |
| 10 | 100 | 50 |  | QPSK | QPSK | 10 | 12 |
| 11 | 100 | 50 |  | 256QAM | 256QAM | 100 | 50 |
| 12 | 100 | 50 |  | 256QAM | QPSK | 100 | 12 |
| 13 | 100 | 100 |  | QPSK | QPSK | 10 | 18 |
| 14 | 100 | 100 |  | 256QAM | 256QAM | 100 | 100 |
| 15 | 100 | 100 |  | 256QAM | QPSK | 100 | 18 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg. Only the combination with the highest NRB_PCC is tested. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBmax-LCRB+1. Note 4: Mapping of NS-value per CC and PCC/SCC selection are specified in Table 6.6.2.2A.2_2.4.1-1. |  |  |  |  |  |  |  |

Table 6.6.2.2A.2_2.4.1-3: Test Configuration Table (network signalled value NS_01/NS_06)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC (Note 3) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg(Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_03) | SCC (NS_07) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 6 | 6 |
| 2 | 25 | 25 |  | 256QAM | 256QAM | 25 | 9 |
| 3 | 25 | 25 |  | QPSK | 256QAM | 6 | 9 |
| 4 | 50 | 50 |  | QPSK | QPSK | 12 | 12 |
| 5 | 50 | 50 |  | 256QAM | 256QAM | 50 | 15 |
| 6 | 50 | 50 |  | QPSK | 256QAM | 12 | 15 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg. Only the combination with the highest NRB_PCC is tested. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBmax-LCRB+1. |  |  |  |  |  |  |  |

Table 6.6.2.2A.2_2.4.1-4: Test Configuration Table (network signalled value NS_01/NS_21)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_01) | SCC (NS_21) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 8 | 8 |
| 2 | 25 | 25 |  | 256QAM | 256QAM | 25 | 25 |
| 3 | 25 | 25 |  | QPSK | 256QAM | 8 | 25 |
| 4 (Note 4) | 50 | 50 |  | QPSK | QPSK | 12 | 12@7 |
| 5 (Note 5) | 50 | 50 |  | QPSK | QPSK | 12 | 12@31 |
| 6 | 50 | 50 |  | 256QAM | 256QAM | 50 | 1 |
| 7 | 50 | 50 |  | QPSK | 256QAM | 12 | 1 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column in Table 6.6.2.2A.2_2.4.1-1. Note 3: For partial RB allocation unless RBstart is explicitly defined, for Low range place test point with RBstart=0, for High range place test point with RBstart=RBmax-LCRB+1. Note 4: Applicable only to Low range frequency testing. Note 5: Applicable only to High range frequency testing. |  |  |  |  |  |  |  |

Table 6.6.2.2A.2_2.4.1-5: Test Configuration Table (network signalled value NS_03/NS_03)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC (Note 3) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg (Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_03) | SCC (NS_03) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 15 | 25 | N/A for this test | QPSK | QPSK | 4 | 6 |
| 2 | 15 | 25 |  | 256QAM | 256QAM | 15 | 25 |
| 3 | 15 | 25 |  | 256QAM | QPSK | 15 | 6 |
| 4 | 25 | 25 |  | QPSK | QPSK | 6 | 6 |
| 5 | 25 | 25 |  | 256QAM | 256QAM | 25 | 25 |
| 6 | 25 | 25 |  | 256QAM | QPSK | 25 | 6 |
| 7 | 50 | 50 |  | QPSK | QPSK | 6 | 6 |
| 8 | 50 | 50 |  | 256QAM | 256QAM | 50 | 50 |
| 9 | 50 | 50 |  | 256QAM | QPSK | 50 | 6 |
| 10 | 100 | 100 |  | QPSK | QPSK | 10 | 10 |
| 11 | 100 | 100 |  | 256QAM | 256QAM | 100 | 100 |
| 12 | 100 | 100 |  | 256QAM | QPSK | 100 | 10 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg. Only the combination with the highest NRB_PCC is tested. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBmax-LCRB+1. |  |  |  |  |  |  |  |

Table 6.6.2.2A.2_2.4.1-6: Test Configuration Table (network signalled value NS_03/NS_06)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC (Note 3) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg(Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_03) | SCC (NS_06) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 15 | 25 | N/A for this test | QPSK | QPSK | 4 | 8 |
| 2 | 15 | 25 |  | 256QAM | 256QAM | 15 | 25 |
| 3 | 15 | 25 |  | 256QAM | QPSK | 15 | 8 |
| 4 | 25 | 15 |  | QPSK | QPSK | 6 | 4 |
| 5 | 25 | 15 |  | 256QAM | 256QAM | 25 | 15 |
| 6 | 25 | 15 |  | 256QAM | QPSK | 25 | 4 |
| 7 | 25 | 25 |  | QPSK | QPSK | 6 | 8 |
| 8 | 25 | 25 |  | 256QAM | 256QAM | 25 | 25 |
| 9 | 25 | 25 |  | 256QAM | QPSK | 25 | 8 |
| 10 | 25 | 50 |  | QPSK | QPSK | 6 | 12 |
| 11 | 25 | 50 |  | 256QAM | 256QAM | 25 | 50 |
| 12 | 25 | 50 |  | 256QAM | QPSK | 25 | 12 |
| 13 | 50 | 50 |  | QPSK | QPSK | 6 | 12 |
| 14 | 50 | 50 |  | 256QAM | 256QAM | 50 | 50 |
| 15 | 50 | 50 |  | 256QAM | QPSK | 50 | 12 |
| 16 | 100 | 50 |  | QPSK | QPSK | 10 | 12 |
| 17 | 100 | 50 |  | 256QAM | 256QAM | 100 | 50 |
| 18 | 100 | 50 |  | 256QAM | QPSK | 100 | 12 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg. Only the combination with the highest NRB_PCC is tested. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBmax-LCRB+1. |  |  |  |  |  |  |  |

Table 6.6.2.2A.2_2.4.1-7: Test Configuration Table (network signalled value NS_03/NS_07)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC (Note 3) |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg(Note 2) |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_03) | SCC (NS_07) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 50 | N/A for this test | QPSK | QPSK | 6 | 6@13 |
| 2 | 25 | 50 |  | 256QAM | 256QAM | 25 | 50 |
| 3 | 25 | 50 |  | QPSK | 256QAM | 6 | 50 |
| 4 | 50 | 50 |  | QPSK | QPSK | 6 | 6@13 |
| 5 | 50 | 50 |  | 256QAM | 256QAM | 50 | 50 |
| 6 | 50 | 50 |  | QPSK | 256QAM | 6 | 50 |
| 7 | 100 | 50 |  | QPSK | QPSK | 10 | 6@13 |
| 8 | 100 | 50 |  | 256QAM | 256QAM | 100 | 50 |
| 9 | 100 | 50 |  | QPSK | 256QAM | 10 | 50 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg. Only the combination with the highest NRB_PCC is tested. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBmax-LCRB+1. |  |  |  |  |  |  |  |

Table 6.6.2.2A.2_2.4.1-8: Test Configuration Table (network signalled value NS_03/NS_21)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_03) | SCC (NS_21) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 6 | 8 |
| 2 | 25 | 25 |  | 256QAM | 256QAM | 25 | 25 |
| 3 | 25 | 25 |  | 256QAM | QPSK | 25 | 8 |
| 4 (Note 4) | 100 | 50 |  | QPSK | QPSK | 10 | 12@7 |
| 5 (Note 5) | 100 | 50 |  | QPSK | QPSK | 10 | 12@31 |
| 6 | 100 | 50 |  | 256QAM | 256QAM | 100 | 1 |
| 7 | 100 | 50 |  | QPSK | 256QAM | 10 | 1 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column in Table 6.6.2.2A.2_2.4.1-1. Note 3: For partial RB allocation unless RBstart is explicitly defined, for Low range place test point with RBstart=0, for High range place test point with RBstart=RBmax-LCRB+1. Note 4: Applicable only to Low range frequency testing. Note 5: Applicable only to High range frequency testing. |  |  |  |  |  |  |  |

Table 6.6.2.2A.2_2.4.1-9: Test Configuration Table (network signalled value NS_06/NS_21)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest and Highest NRBagg |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |
|  | PCC (NS_06) | SCC (NS_21) | PCC & SCC RB allocation | PCC | SCC | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | QPSK | 8 | 8 |
| 2 | 25 | 25 |  | 256QAM | 256QAM | 25 | 25 |
| 3 | 25 | 25 |  | 256QAM | QPSK | 25 | 8 |
| 4 (Note 4) | 50 | 50 |  | QPSK | QPSK | 12 | 12@7 |
| 5 (Note 5) | 50 | 50 |  | QPSK | QPSK | 12 | 12@31 |
| 6 | 50 | 50 |  | 256QAM | 256QAM | 50 | 1 |
| 7 | 50 | 50 |  | QPSK | 256QAM | 12 | 1 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: For simplifying test, PCC is mapped to the first band of the combination as listed in Uplink CA Configuration column in Table 6.6.2.2A.2_2.4.1-1. Note 3: For partial RB allocation unless RBstart is explicitly defined, for Low range place test point with RBstart=0, for High range place test point with RBstart=RBmax-LCRB+1. Note 4: Applicable only to Low range frequency testing. Note 5: Applicable only to High range frequency testing. |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] clause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.2A.2_2.4.1-2 to 6.6.2.2A.2_2.4.1-9.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.2A.2_2.4.3.

6.6.2.2A.2_2.4.2	Test procedure

Same test procedure as in clause 6.6.2.2A.2.4.2.

6.6.2.2A.2_2.4.3	Message contents

Same message contents as in clause 6.6.2.2A.2.4.3.

6.6.2.2A.2_2.5	Test Requirements

Same test requirements as in clause 6.6.2.2A.2.5.

##### 6.6.2.2A.3	Additional Spectrum Emission Mask for CA (intra-band non-contiguous DL CA and UL CA)

6.6.2.2A.3.1	Test purpose

To verify that the power of any UE emission shall not exceed specified lever for the specified channel bandwidth for CA under the deployment scenarios where additional requirements are specified.

6.6.2.2A.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 11 and forward that support intra-band non-contiguous DL CA and UL CA.

6.6.2.2A.3.3	Minimum conformance requirements

Minimum conformance requirements are specified in section 6.2.4A.3.1.

6.6.2.2A.3.4	Test description

6.6.2.2A.3.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-3. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.2A.3.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.4A.3.4.1-1: Test Configuration Table (network signalled value CA_NC_NS_01)

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Refer to test points A: Low Wgap, Maximum Wgap |  |  |  |  |
| Test CC Combination setting (NRB) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  | Refer to test point Test only test points with Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  | Wgap [MHz] | DL Allocation |  | UL Allocation |  |  |  |
|  | PCC NRB | SCCs NRB |  | CC MOD | PCC & SCC RB allocation | CC MOD | PCC NRB_alloc (LCRB @ RBstart) | CC MOD | SCC NRB_alloc (LCRB @ RBstart) |
| Test Parameters for CA_4A-4A Configurations |  |  |  |  |  |  |  |  |  |
| 1 | 25 | 25 | 35 | N/A | N/A | 16QAM | P_1@0 | 16QAM | S_1@24 |
| 2 | 25 | 25 | 35 | N/A | N/A | 16QAM | P_25@0 | 16QAM | S_25@0 |
| 3 | 25 | 25 | 5 | N/A | N/A | 16QAM | P_1@0 | 16QAM | S_1@24 |
| 4 | 25 | 25 | 5 | N/A | N/A | 16QAM | P_25@0 | 16QAM | S_25@0 |
| 5 | 100 | 100 | 5 | N/A | N/A | 16QAM | P_1@0 | 16QAM | S_1@99 |
| 6 | 100 | 100 | 5 | N/A | N/A | 16QAM | P_25@0 | 16QAM | S_25@75 |
| 7 | 100 | 100 | 5 | N/A | N/A | 16QAM | P_100@0 | 16QAM | S_100@0 |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] clause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.2A.3.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.2A.3.4.3.

6.6.2.2A.3.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.6.2.2A.3.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.6.2.2A.3.4.1-1 on both PCC and SCC. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

6.	Measure the mean power over all component carriers in the CA configuration of the radio access mode, which shall meet the requirements described in Table 6.2.3A.3.5-1. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

7.	Measure the power of the transmitted signal for PCC with a measurement filter of bandwidths according to tables 6.6.2.2A.3.5.1-1 to 6.6.2.2A.3.5.1-2 as applicable and using a rms detector. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

8.	Measure the power of the transmitted signal for SCC with a measurement filter of bandwidths according to tables 6.6.2.2A.3.5.1-1 to 6.6.2.2A.3.5.1-2 as applicable and using a rms detector. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.6.2.2A.3.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6. The following exceptions apply for each network signalled value. These exceptions indicate that the UE shall meet the additional spectrum emission requirement for a specific deployment scenario.

6.6.2.2A.3.4.3.2	Message contents exceptions (network signalled value "CA_NC_NS_01")

1.	Information element additionalSpectrumEmissionSCell-r10 is set to CA_NC_NS_01. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages.

Table 6.6.2.2A.3.4.3.2-1: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "CA_NC_NS_01"

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 1 (CA_NC_NS_01) |  |  |

6.6.2.2A.3.5	Test Requirements

The measured UE mean power with the aggregated channel bandwidth as specified in clause 5.4.2A, derived in step 6 , shall fulfil requirements in Table 6.2.3A.3.5-1,

and

The power of any UE emission per component carrier derived in step 7 and 8 shall not exceed the levels specified in Tables 6.6.2.2A.3.5.1-1 to 6.6.2.2A.3.5.1-2. The spectrum emission mask requirement is defined as a composite spectrum emissions mask, as defined in section 6.6.2.1A.0.

Table 6.6.2.2A.3.5-1: Additional requirements (network signalled value " CA_NC_NS_01"), E-UTRA bands ≤ 3GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.5 | -11.5 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 1-2.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 |  |  |  |  | 1 MHz |
| 6-10 |  |  | -23.5 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.5 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.5 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_03 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

Table 6.6.2.2A.3.5-2: Additional requirements (network signalled value " CA_NC_NS_01"), 3GHz < E-UTRA bands ≤ 4.2GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.2 | -11.2 | -13.2 | -16.2 | -18.2 | -19.2 | 30 kHz |
| 1-2.5 | -11.2 | -11.2 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 2.5-2.8 | -23.2 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.2 |  |  |  |  | 1 MHz |
| 6-10 |  |  | -23.2 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.2 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.2 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.2 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_03 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

NOTE:	(only for emission measurement) As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

##### 6.6.2.2A.4	Additional Spectrum Emission Mask for CA (3UL CA)

6.6.2.2A.4.1	Test purpose

To verify that the power of any UE emission shall not exceed specified lever for the specified channel bandwidth for CA under the deployment scenarios where additional requirements are specified.

6.6.2.2A.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 13 and forward that support intra-band contiguous or inter-band and 3UL CA.

6.6.2.2A.4.3	Minimum conformance requirements

The minimum requirements are the same as in section 6.6.2.2.3 per component carrier.

6.6.2.2A.4.4	Test description

6.6.2.2A.4.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-3. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.2A.4.4.1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.2A.4.4.1-1: Test Configuration Table for intra-band contiguous CA

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  |  | C: Low and High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  |  | As in Table 6.6.2.2A.1.3.1-1 |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |  |
|  | PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc |  | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 100 | 50 | N/A | QPSK | 10 |  | P_10@20 | S_0@0 | S_0@0 |  |
| 2 | 100 | 50 |  | QPSK | 70 |  | P_50@50 | S_10@0 | S_10@0 |  |
| 3 | 100 | 50 |  | 16QAM | 30 |  | P_0@0 | S_15@0 | S_15@0 |  |
| 4 | 100 | 50 |  | QPSK | 3 |  | P_1@0 | S_1@49 | S_1@49 |  |
| 5 | 75 | 75 |  | QPSK | 10 |  | P_10@20 | S_0@0 | S_0@0 |  |
| 6 | 75 | 75 |  | QPSK | 120 |  | P_30@45 | S_45@0 | S_45@0 |  |
| 7 | 75 | 75 |  | QPSK | 3 |  | P_1@0 | S_1@74 | S_1@74 |  |
| 8 | 100 | 75 |  | QPSK | 10 |  | P_10@20 | S_0@0 | S_0@0 |  |
| 9 | 100 | 75 |  | QPSK | 110 |  | P_50@50 | S_30@0 | S_30@0 |  |
| 10 | 100 | 75 |  | 16QAM | 40 |  | P_0@0 | S_20@15 | S_20@15 |  |
| 11 | 100 | 75 |  | QPSK | 3 |  | P_1@0 | S_1@74 | S_1@74 |  |
| 12 | 100 | 100 |  | QPSK | 10 |  | P_10@25 | S_0@0 | S_0@0 |  |
| 13 | 100 | 100 |  | QPSK | 140 |  | P_40@60 | S_50@0 | S_50@0 |  |
| 14 | 100 | 100 |  | 16QAM | 30 |  | P_0@0 | S_15@40 | S_15@40 |  |
| 15 | 100 | 100 |  | 16QAM | 40 |  | P_0@0 | S_20@30 | S_20@30 |  |
| 16 | 100 | 100 |  | QPSK | 3 |  | P_1@0 | S_1@99 | S_1@99 |  |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1 |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] clause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.2A.4.4.1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.2A.4.4.3.

6.6.2.2A.4.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.6.2.2A.4.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.6.2.2A.4.4.1 on both PCC and SCC. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

6.	Measure the mean power over all component carriers in the CA configuration of the radio access mode, which shall meet the requirements described in Table 6.2.3A.3.5-1. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

7.	Measure the power of the transmitted signal for PCC with a measurement filter of bandwidths according to tables 6.6.2.2A.4.5.1 as applicable and using a rms detector. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

8.	Measure the power of the transmitted signal for SCC with a measurement filter of bandwidths according to tables 6.6.2.2A.4.5.1 as applicable and using a rms detector. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.6.2.2A.4.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6. The following exceptions apply for each network signalled value. These exceptions indicate that the UE shall meet the additional spectrum emission requirement for a specific deployment scenario.

6.6.2.2A.4.5	Test Requirements

The measured UE mean power with the aggregated channel bandwidth as specified in clause 5.4.2A, derived in step 6 , shall fulfil requirements in Table 6.6.2.2A.4.5,and

- the power of any UE emission shall not exceed the levels specified in Table 6.6.2.2A.1.5.1-1.

Table 6.6.2.2A.4.5-1: Additional requirements

| Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 50+100RB (29.9 MHz) | 75+75B (30 MHz) | 75+100RB (34.85 MHz) | 100+100RB (39.8 MHz) | Measurement bandwidth |
| 0-1 | -21 | -21 | -22 | -22.5 | 30 kHz |
| 1-5.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 5.5-34.9 | -23.5 | -23.5 | -23.5 | -23.5 | 1 MHz |
| 34.9-35 |  | -23.5 | -23.5 | -23.5 | 1 MHz |
| 35-39.85 |  |  | -23.5 | -23.5 | 1 MHz |
| 39.85-44.8 |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value CA_NS_04 as defined in TS 36.101 [2] subclause 6.6.2.2A Table 6.2.2.2A-1. |  |  |  |  |  |

##### 6.6.2.2A.5	Additional Spectrum Emission Mask for CA (4UL CA)

Editor’s Note: This test case is incomplete. The following items are missing or are incomplete:

- Test configuration table is TBD, because A-MPR test case for 4UL CA is incomplete

- A-MPR related test requirements are TBD, because A-MPR test case for 4UL CA is incomplete

6.6.2.2A.5.1	Test purpose

To verify that the power of any UE emission shall not exceed specified lever for the specified channel bandwidth for CA under the deployment scenarios where additional requirements are specified.

6.6.2.2A.5.2	Test applicability

This test case applies to all types of E-UTRA UE release 11 and forward that support inter-band and 4UL CA.

6.6.2.2A.5.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.6.2.2A.1.3

6.6.2.2A.5.4	Test description

6.6.2.2A.5.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-2. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.2A.5.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.2A.5.4.1-1: Test Configuration Table (network signalled value CA_NS_04/CA_NS_08)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  |  |  |  |  | NC |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  |  |  |  |  | Low and High range for both PCC and SCC |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  |  | Lowest and Highest NRBagg |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  |  |  | DL Allocation | CC MOD |  |  |  | UL Allocation |  |  |  |
|  | PCC | SCC1 | SCC2 | SCC3 | PCC & SCC RB allocation | PCC | SCC1 | SCC2 | SCC3 | PCC & SCC RB allocations |  |  |  |
|  | NRB | NRB | NRB | NRB |  |  |  |  |  |  |  |  |  |
| 1 | TBD | TBD | TBD | TBD | N/A for this test | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| 2 | TBD | TBD | TBD | TBD |  | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| 3 | TBD | TBD | TBD | TBD |  | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration. which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: Lower limit is assuming ΔTIB,c is zero. If non-zero. PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure group A.55 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.6.2.2A.5.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.2A.5.4.3.

6.6.2.2A.1.4.2	Test procedure

1.	Configure SCCs according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCCs as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.6.2.2A.5.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.6.2.2A.5.4.1-1 on both PCC and SCC. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

6.	Measure the mean power over all component carriers in the CA configuration of the radio access mode, which shall meet the requirements described in Table TBD. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

7.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to table 6.6.2.2A.1.5.1-1. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.6.2.2A.5.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6. The following exceptions apply for each network signalled value.

6.6.2.2A.1.4.3.1	Message contents exceptions (network signalled value "CA_NS_04")

1.	Information element additionalSpectrumEmissionSCell-r10 is set to CA_NS_04. This can be set in the RadioResourceConfigCommonSCell-r10-DEFAULT as part of the common RRC messages. This exception indicates that the UE shall meet the additional spectrum emission requirement for a specific deployment scenario.

Table 6.6.2.2A.5.4.3.1-1: RadioResourceConfigCommonSCell-r10-DEFAULT: Additional spectrum emission test requirement for "CA_NS_04"

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmissionSCell-r10 | 4 (CA_NS_04) |  |  |

6.6.2.2A.5.5.1	Test requirement for CA (network signalled value "CA_NS_04")

When "CA_NS_04" is indicated in the cell the measured UE mean power with the aggregated channel bandwidth as specified in clause 5.4.2A, derived in step 6 , shall fulfil requirements in Table TBD as appropriate, and the power of any UE emission shall not exceed the levels specified in Table 6.6.2.2A.1.5.1-1.

Table 6.6.2.2A.1.5.1-1: Additional requirements

| Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 50+75RB (24.75 MHz) | 25+100RB (24.95 MHz) | 50+100RB (29.9 MHz) | 75+75RB (30 MHz) | 75+100RB (34.85 MHz) | 100+100RB (39.8 MHz) | Measurement bandwidth |
| 0-1 | -20.5 | -20.5 | -21 | -21.5 | -22 | -22.5 | 30 kHz |
| 1-5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | 1 MHz |
| 5-22.95 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 22.95-23.25 | -11.5 | -23.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 23.25-27.9 | -23.5 | -23.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 27.9-28.5 | -23.5 | -23.5 | -23.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 28.5-29.75 | -23.5 | -23.5 | -23.5 | -23.5 | -11.5 | -11.5 | 1 MHz |
| 29.75-29.95 |  | -23.5 | -23.5 | -23.5 | -11.5 | -11.5 | 1 MHz |
| 29.95-32.85 |  |  | -23.5 | -23.5 | -11.5 | -11.5 | 1 MHz |
| 32.85-34.9 |  |  | -23.5 | -23.5 | -23.5 | -11.5 | 1 MHz |
| 34.9-35 |  |  |  | -23.5 | -23.5 | -11.5 | 1 MHz |
| 35-37.8 |  |  |  |  | -23.5 | -11.5 | 1 MHz |
| 37.8-39.85 |  |  |  |  | -23.5 | -23.5 | 1 MHz |
| 39.85-44.8 |  |  |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel. Note 4: Above SEM requirement applies to bands corresponding to network signalling value CA_NS_04 as defined in TS 36.101 [2] subclause 6.6.2.2A Table 6.6.2.2A-1. |  |  |  |  |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

#### 6.6.2.2B	Additional Spectrum Emission Mask for UL-MIMO

##### 6.6.2.2B.1	Test purpose

To verify that the power of any UE emission at each transmit antenna shall not exceed specified lever for the specified channel bandwidth under the deployment scenarios where additional requirements are specified.

##### 6.6.2.2B.2	Test applicability

This test case applies to all types of E-UTRA UE release 10 and forward that support UL-MIMO

##### 6.6.2.2B.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.2.2.3.

##### 6.6.2.2B.4	Test description

6.6.2.2B.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Tables 6.6.2.2B.4.1-1 to 6.6.2.2B.4.1-7.The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2 respectively. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.2B.4.1-1: Test Configuration Table (network signalled value "NS_03", “NS_11”, and “NS_20”)

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] clause 4.1) |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS 36.508 [7] clause 4.3.1) |  |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] clause 4.3.1) |  |  |  | Lowest, 5MHz, 10MHz, Highest |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |  |
| Ch BW | Mod'n | RB allocation |  | Mod'n | RB allocation |  |
|  |  | FDD | TDD |  | FDD | TDD |
| 1.4MHz | N/A for Additional Spectrum Emission Mask testing. |  |  | QPSK | 6 | 6 |
| 1.4MHz |  |  |  | QPSK | 5 | 5 |
| 1.4MHz |  |  |  | 16QAM | 5 | 5 |
| 3MHz |  |  |  | QPSK | 15 | 15 |
| 3MHz |  |  |  | QPSK | 4 | 4 |
| 3MHz |  |  |  | 16QAM | 15 | 15 |
| 3MHz |  |  |  | 16QAM | 4 | 4 |
| 5MHz |  |  |  | QPSK | 25 | 25 |
| 5MHz |  |  |  | QPSK | 8 | 8 |
| 5MHz |  |  |  | QPSK | 6 | 6 |
| 5MHz |  |  |  | 16QAM | 25 | 25 |
| 5MHz |  |  |  | 16QAM | 8 | 8 |
| 10MHz |  |  |  | QPSK | 50 | 50 |
| 10MHz |  |  |  | QPSK | 12 | 12 |
| 10MHz |  |  |  | QPSK | 6 | 6 |
| 10MHz |  |  |  | 16QAM | 50 (Note 3) | 50 (Note 3) |
| 10MHz |  |  |  | 16QAM | 12 | 12 |
| 15MHz |  |  |  | QPSK | 75 | 75 |
| 15MHz |  |  |  | QPSK | 16 | 16 |
| 15MHz |  |  |  | QPSK | 8 | 8 |
| 15MHz |  |  |  | 16QAM | 75 (Note 3) | 75 (Note 3) |
| 15MHz |  |  |  | 16QAM | 16 | 16 |
| 20MHz |  |  |  | QPSK | 100 | 100 |
| 20MHz |  |  |  | QPSK | 18 | 18 |
| 20MHz |  |  |  | QPSK | 10 | 10 |
| 20MHz |  |  |  | 16QAM | 100 (Note 3) | 100 (Note 3) |
| 20MHz |  |  |  | 16QAM | 18 | 18 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 3: Applies only for UE-Categories [FFS]. |  |  |  |  |  |  |

Table 6.6.2.2B.4.1-2: Test Configuration Table (network signalled value "NS_04")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range Additional test frequencies for Rel-12 to Rel-13 and bit 2 of modifiedMPRbehavior = 0: For 5 MHz bandwidth: 2499.6 MHz, NUL= 39686 For 10 MHz bandwidth: 2504.1 MHz, NUL= 39731 For 15 MHz bandwidth: 2510.9 MHz, NUL= 39799 For 20 MHz bandwidth: 2517.6 MHz, NUL= 39866 Additional test frequencies for Rel-12 and later and bit 2 of modifiedMPRbehavior = 1: For 5 MHz bandwidth: 2500.6 MHz, NUL= 39696 For 10 MHz bandwidth: 2504.1 MHz, NUL= 39731 For 15 MHz bandwidth: 2510.9 MHz, NUL= 39799 For 20 MHz bandwidth: 2517.6 MHz, NUL= 39866 |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 5MHz, 10 MHz, 15 MHz, 20MHz |  |  |
| Test Parameters for NS_04 A-MPR |  |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configura-tion ID | Ch BW | Mod'n | RB allocation TDD | Mod'n | RB allocation TDD | RBstart TDD |
| 1 (Note 6) | 5MHz | N/A for A-MPR testing |  | QPSK | 25 | 0 |
| 2 (Note 6) | 5MHz |  |  | QPSK | 8 | Note 3 |
| 3(Note 6) | 5MHz |  |  | QPSK | 6 | Note 3 |
| 4 (Note 6) | 5MHz |  |  | 16QAM (Note 5) | 25 | 0 |
| 5 (Note 6) | 5MHz |  |  | 16QAM (Note 5) | 8 | Note 3 |
| 6 (Note 6) | 10MHz |  |  | QPSK | 1 | 0 |
| 7 (Note 6) | 10MHz |  |  | QPSK | 12 | 0 |
| 8 (Note 6) | 10MHz |  |  | QPSK | 50 | 0 |
| 9 Note 6) | 10MHz |  |  | 16QAM (Note 5) | 50 (Note 4) | 0 |
| 10 (Note 6) | 10MHz |  |  | QPSK | 24 | 13 |
| 11 (Note 6) | 10MHz |  |  | 16QAM (Note 5) | 24 | 13 |
| 12 (Note 6) | 10MHz |  |  | QPSK | 36 | 13 |
| 13 (Note 6) | 10MHz |  |  | QPSK | 12 | 37 |
| 14 (Note 6) | 10MHz |  |  | QPSK | 1 | 49 |
| 15 (Note 6) | 15MHz |  |  | QPSK | 1 | 0 |
| 16 (Note 6) | 15MHz |  |  | QPSK | 16 | 0 |
| 17 (Note 6) | 15MHz |  |  | QPSK | 75 | 0 |
| 18 (Note 6) | 15MHz |  |  | 16QAM (Note 5) | 75 (Note 4) | 0 |
| 19 (Note 6) | 15MHz |  |  | QPSK | 36 | 19 |
| 20 (Note 6) | 15MHz |  |  | 16QAM (Note 5) | 36 (Note 4) | 19 |
| 21 (Note 6) | 15MHz |  |  | QPSK | 50 | 19 |
| 22 (Note 6) | 15MHz |  |  | QPSK | 18 | 56 |
| 23 (Note 6) | 15MHz |  |  | QPSK | 1 | 74 |
| 24 (Note 6) | 20MHz |  |  | QPSK | 1 | 0 |
| 25 (Note 6) | 20MHz |  |  | QPSK | 18 | 0 |
| 26 (Note 6) | 20MHz |  |  | QPSK | 100 | 0 |
| 27 (Note 6) | 20MHz |  |  | 16QAM (Note 5) | 100 (Note 4) | 0 |
| 28 (Note 6) | 20MHz |  |  | QPSK | 50 | 25 |
| 29 (Note 6) | 20MHz |  |  | 16QAM (Note 5) | 50 (Note 4) | 25 |
| 30 (Note 6) | 20MHz |  |  | QPSK | 75 | 25 |
| 31 (Note 6) | 20MHz |  |  | QPSK | 25 | 75 |
| 32 (Note 6) | 20MHz |  |  | QPSK | 1 | 99 |
| 33 (Note 8) | 5MHz |  |  | QPSK | 1 | 0 |
| 34 (Note 8) | 5MHz |  |  | QPSK | 16 | 9 |
| 35 (Note 9) | 5MHz |  |  | QPSK | 25 | 0 |
| 36 (Note 8) | 5MHz |  |  | 16QAM (Note 5) | 16 | 9 |
| 37 (Note 9) | 5MHz |  |  | 16QAM (Note 5) | 25 | 0 |
| 38 (Note 8) | 10MHz |  |  | QPSK | 1 | 0 |
| 39 (Note 8) | 10MHz |  |  | QPSK | 1 | 49 |
| 40 (Note 8) | 10MHz |  |  | QPSK | 24 | 0 |
| 41 (Note 8) | 10MHz |  |  | QPSK | 40 | 10 |
| 42 (Note 7) | 10MHz |  |  | QPSK | 50 | 0 |
| 43 (Note 7) | 10MHz |  |  | 16QAM (Note 5) | 50 | 0 |
| 44 (Note 8) | 15MHz |  |  | QPSK | 1 | 0 |
| 45 (Note 8) | 15MHz |  |  | QPSK | 1 | 74 |
| 46 (Note 8) | 15MHz |  |  | QPSK | 32 | 0 |
| 47 (Note 8) | 15MHz |  |  | QPSK | 60 | 15 |
| 48 (Note 7) | 15MHz |  |  | QPSK | 75 | 0 |
| 49 (Note 7) | 15MHz |  |  | 16QAM (Note 5) | 75 | 0 |
| 50 (Note 8) | 20MHz |  |  | QPSK | 1 | 0 |
| 51 (Note 8) | 20MHz |  |  | QPSK | 1 | 99 |
| 52 (Note 8) | 20MHz |  |  | QPSK | 36 | 0 |
| 53 (Note 8) | 20MHz |  |  | QPSK | 75 | 25 |
| 54 (Note 7) | 20MHz |  |  | QPSK | 100 | 0 |
| 55 (Note 7) | 20MHz |  |  | 16QAM (Note 5) | 100 | 0 |
| Note 1: Test Channel Bandwidths are checked separately for E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The configuration ID will be used to map the applicable Test Configuration to be corresponding Test Requirement in subclause 6.2.4 as not all combinations are necessarily required based on the applicability of the UE. Note 3: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 4: Applies only for UE-Categories ≥2. Note 5: Void. Note 6: Applies only for Rel-11 and earlier. Note 7: Applies only for Rel-12 and later. Note 8: Applies only for Rel-12 and later and Low Range. Note 9: Applies only for Rel-12 and later and Mid, High Range and additional test frequencies. |  |  |  |  |  |  |

Table 6.6.2.2B.4.1-3: Test Configuration Table (network signalled value "NS_06")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] clause 4.1) |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS 36.508 [7] clause 4.3.1) |  |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] clause 4.3.1) |  |  |  | Lowest, 5MHz, 10MHz, Highest |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |  |
| Ch BW | Mod'n | RB allocation |  | Mod'n | RB allocation |  |
|  |  | FDD | TDD |  | FDD | TDD |
| 1.4MHz | N/A for Additional Spectrum Emission Mask testing. |  |  | QPSK | 6 | NA |
| 1.4MHz |  |  |  | QPSK | 5 |  |
| 1.4MHz |  |  |  | 16QAM | 5 |  |
| 3MHz |  |  |  | QPSK | 15 |  |
| 3MHz |  |  |  | QPSK | 4 |  |
| 3MHz |  |  |  | 16QAM | 4 |  |
| 5MHz |  |  |  | QPSK | 25 |  |
| 5MHz |  |  |  | QPSK | 8 |  |
| 5MHz |  |  |  | 16QAM | 8 |  |
| 10MHz |  |  |  | QPSK | 50 |  |
| 10MHz |  |  |  | QPSK | 12 |  |
| 10MHz |  |  |  | 16QAM | 12 |  |
| 15MHz |  |  |  | QPSK | 75 |  |
| 15MHz |  |  |  | QPSK | 16 |  |
| 15MHz |  |  |  | 16QAM | 16 |  |
| 20MHz |  |  |  | QPSK | 100 |  |
| 20MHz |  |  |  | QPSK | 18 |  |
| 20MHz |  |  |  | 16QAM | 18 |  |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. |  |  |  |  |  |  |

Table 6.6.2.2B.4.1-4: Test Configuration Table (network signalled value "NS_07")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] clause 4.1) |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS 36.508 [7] clause 4.3.1) |  |  |  | Mid range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] clause 4.3.1) |  |  |  | 10MHz |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Test Number | Ch BW | Mod'n | RB allocation | Mod'n | RB allocation | RBstart |
| 1 | 10MHz | N/A for Additional Spectrum Emission Mask testing. |  | QPSK | 1 | 0 |
| 2 | 10MHz |  |  | QPSK | 8 | 0 |
| 3 | 10MHz |  |  | QPSK | 6 | 13 |
| 4 | 10MHz |  |  | QPSK | 20 | 13 |
| 5 | 10MHz |  |  | QPSK | 12 | 13 |
| 6 | 10MHz |  |  | 16QAM | 36 (Note 1) | 13 |
| 7 | 10MHz |  |  | QPSK | 16 | 19 |
| 8 | 10MHz |  |  | QPSK | 12 | 19 |
| 9 | 10MHz |  |  | 16QAM | 16 | 19 |
| 10 | 10MHz |  |  | QPSK | 30 | 19 |
| 11 | 10MHz |  |  | 16QAM | 30 (Note 1) | 19 |
| 12 | 10MHz |  |  | QPSK | 6 | 43 |
| 13 | 10MHz |  |  | QPSK | 2 | 48 |
| 14 | 10MHz |  |  | QPSK | 50 | 0 |
| 15 | 10MHz |  |  | QPSK | 12 | 0 |
| 16 | 10MHz |  |  | 16QAM | 50 (Note 1) | 0 |
| Note 1: Applies only for UE-Categories [FFS]. |  |  |  |  |  |  |

Table 6.6.2.2B.4.1-5: Test Configuration Table (network signalled value "NS_21")

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  |  |  | NC |  |  |  |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  |  |  |  |  |  | Low range or High range |  |  |  |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  |  |  | 5 MHz, 10 MHz |  |  |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Downlink Configuration |  |  | Uplink Configuration |  |  |  |  |  |
| Configuration ID |  | Ch BW |  | Mod'n |  | RB allocation | Mod'n |  | RB allocation FDD |  | RBstart FDD |  |
| 1 (note 2) |  | 5 MHz |  |  |  |  | QPSK |  | 1 |  | 0 |  |
| 2 (note 2) |  | 5 MHz |  |  |  |  | QPSK |  | 25 |  | 0 |  |
| 3 (note 2) |  | 10 MHz |  |  |  |  | QPSK |  | 1 |  | 0 |  |
| 4 (note 3) |  | 10 MHz |  |  |  |  | QPSK |  | 1 |  | 49 |  |
| 5 (note 2) |  | 10 MHz |  |  |  |  | QPSK |  | 15 |  | 0 |  |
| 6 (note 3) |  | 10 MHz |  |  |  |  | QPSK |  | 3 |  | 47 |  |
| 7 (note 3) |  | 10 MHz |  |  |  |  | QPSK |  | 50 |  | 0 |  |
| 8 (note 1, 3) |  | 10 MHz |  |  |  |  | 16QAM |  | 50 |  | 0 |  |
| Note 1: Applies only for UE-Categories ≥2. Note 2: Applicable only to low range frequency testing. Note 3: Applicable only to high range frequency testing. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.6.2.2B.4.1-6: Test Configuration Table (network signalled value "NS_27")

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  | Normal |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | Refer to Frequency column in test parameters |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | 5MHz, 10MHz, 15MHz, 20MHz |  |  |
| Test Parameters for NS_27 A-MPR |  |  |  |  |  |  |  |
|  |  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID | Frequency | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD | RBstart FDD |
| 1 | Low/Mid | 5MHz | N/A for A-MPR testing |  | QPSK | 1 | 0 |
| 2 | High | 5MHz |  |  | QPSK | 1 | 24 |
| 3 | Low/Mid/High | 5MHz |  |  | QPSK | 25 | 0 |
| 4 | Low/Mid/High | 5MHz |  |  | 16QAM | 25 | 0 |
| 5 | Low/Mid | 10MHz |  |  | QPSK | 1 | 0 |
| 6 | High | 10MHz |  |  | QPSK | 1 | 49 |
| 7 | Low/Mid/High | 10MHz |  |  | QPSK | 50 | 0 |
| 8 (Note 2) | Low/Mid/High | 10MHz |  |  | 16QAM | 50 | 0 |
| 9 | Low/Mid | 15MHz |  |  | QPSK | 1 | 0 |
| 10 | Low | 15MHz |  |  | QPSK | 1 | 7 |
| 11 | Low/Mid | 15MHz |  |  | QPSK | 16 | 0 |
| 12 | High | 15MHz |  |  | QPSK | 1 | 74 |
| 13 | High | 15MHz |  |  | QPSK | 1 | 67 |
| 14 | High | 15MHz |  |  | QPSK | 16 | 74 |
| 15 | Low/Mid/High | 15MHz |  |  | QPSK | 75 | 0 |
| 16 (Note 2) | Low/Mid/High | 15MHz |  |  | 16QAM | 75 | 0 |
| 17 | Low/Mid | 20MHz |  |  | QPSK | 1 | 0 |
| 18 | Low | 20MHz |  |  | QPSK | 1 | 13 |
| 19 | Low | 20MHz |  |  | QPSK | 1 | 16 |
| 20 | Low/Mid | 20MHz |  |  | QPSK | 21 | 0 |
| 21 | High | 20MHz |  |  | QPSK | 1 | 99 |
| 22 | High | 20MHz |  |  | QPSK | 1 | 86 |
| 23 | High | 20MHz |  |  | QPSK | 1 | 83 |
| 24 | High | 20MHz |  |  | QPSK | 21 | 0 |
| 25 | Low/Mid/High | 20MHz |  |  | QPSK | 100 | 0 |
| 26 (Note 2) | Low/Mid/High | 20MHz |  |  | 16QAM | 100 | 0 |
| Note 1: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4B.5 as not all combinations are necessarily required based on the applicability of the UE. Note 2: Applies only for UE-Categories ≥2. Note 3: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |  |  |

Table 6.6.2.2B.4.1-7: Test Configuration Table (network signalled value "NS_35")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] clause 4.1) |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS 36.508 [7] clause 4.3.1) |  |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] clause 4.3.1) |  |  |  | Lowest, 5MHz, 10MHz, Highest |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |  |
| Ch BW | Mod'n | RB allocation |  | Mod'n | RB allocation |  |
|  |  | FDD | TDD |  | FDD | TDD |
| 5MHz | N/A for Additional Spectrum Emission Mask testing. |  |  | QPSK | 25 |  |
| 5MHz |  |  |  | QPSK | 8 |  |
| 5MHz |  |  |  | 16QAM | 8 |  |
| 10MHz |  |  |  | QPSK | 50 |  |
| 10MHz |  |  |  | QPSK | 12 |  |
| 10MHz |  |  |  | 16QAM | 12 |  |
| 15MHz |  |  |  | QPSK | 75 |  |
| 15MHz |  |  |  | QPSK | 16 |  |
| 15MHz |  |  |  | 16QAM | 16 |  |
| 20MHz |  |  |  | QPSK | 100 |  |
| 20MHz |  |  |  | QPSK | 18 |  |
| 20MHz |  |  |  | 16QAM | 18 |  |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure A.28.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] clause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Tables 6.6.2.2B.4.1-1 to 6.6.2.2B.4.1-7.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.2B.4.3.

6.6.2.2B.4.2	Test procedure

Same test procedure as defined in clause 6.6.2.2.4.2.

6.6.2.2B.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6, with the same exceptions as in clause 6.6.2.2.4.3.

##### 6.6.2.2B.5	Test requirements

Same test requirements as defined in clause 6.6.2.2.5.

#### 6.6.2.2E	Additional Spectrum Emission Mask for UE category 0

##### 6.6.2.2E.1	Test purpose

Same test purpose as in clause 6.6.2.2.1.

##### 6.6.2.2E.2	Test applicability

This test case applies to all types of E-UTRA UE release 12 and forward of UE category 0.

##### 6.6.2.2E.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.6.2.2.3

##### 6.6.2.2E.4	Test description

6.6.2.2E.4.1	Initial conditions

Same initial conditions as in clause 6.6.2.2.4.1 with following exceptions for HD-FDD:

-	Instead of Tables 6.6.2.2.4.1-1 through 6.6.2.2.4.1-10  use Table 6.6.2.2E.4.1-1 through 6.6.2.2E.4.1-4.

-	Connect SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.3 using only main UE Tx/Rx antenna.

Table 6.6.2.2E.4.1-1: Test Configuration Table (network signalled value "NS_03")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Lowest, 5MHz, 10MHz, Highest |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |  |
| Ch BW | Mod'n | RB allocation |  | Mod'n | RB allocation |  |
|  |  | FDD | TDD |  | FDD and HD-FDD | TDD |
| 1.4MHz | N/A for Additional Spectrum Emission Mask testing. |  |  | QPSK | 6 | 6 |
| 1.4MHz |  |  |  | QPSK | 5 | 5 |
| 1.4MHz |  |  |  | 16QAM | 5 | 5 |
| 3MHz |  |  |  | QPSK | 15 | 15 |
| 3MHz |  |  |  | QPSK | 4 | 4 |
| 3MHz |  |  |  | 16QAM | 5 | 5 |
| 3MHz |  |  |  | 16QAM | 4 | 4 |
| 5MHz |  |  |  | QPSK | 25 | 25 |
| 5MHz |  |  |  | QPSK | 8 | 8 |
| 5MHz |  |  |  | QPSK | 6 | 6 |
| 5MHz |  |  |  | 16QAM | 5 | 5 |
| 10MHz |  |  |  | QPSK | 36 | 36 |
| 10MHz |  |  |  | QPSK | 12 | 12 |
| 10MHz |  |  |  | QPSK | 6 | 6 |
| 10MHz |  |  |  | 16QAM | 5 | 5 |
| 15MHz |  |  |  | QPSK | 36 | 36 |
| 15MHz |  |  |  | QPSK | 16 | 16 |
| 15MHz |  |  |  | QPSK | 8 | 8 |
| 15MHz |  |  |  | 16QAM | 5 | 5 |
| 20MHz |  |  |  | QPSK | 36 | 36 |
| 20MHz |  |  |  | QPSK | 18 | 18 |
| 20MHz |  |  |  | QPSK | 10 | 10 |
| 20MHz |  |  |  | 16QAM | 5 | 5 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. |  |  |  |  |  |  |

Table 6.6.2.2E.4.1-2: Test Configuration Table (network signalled value "NS_04")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 5MHz, 10 MHz, 15 MHz, 20MHz |  |  |
| Test Parameters for NS_04 A-MPR |  |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID | Ch BW | Mod'n | RB allocation TDD | Mod'n | RB allocation TDD | RBstart TDD |
| 1 | 5MHz | N/A for Additional Spectrum Emission Mask testing |  | QPSK | 25 | Note 2 |
| 2 | 5MHz |  |  | QPSK | 8 | Note 2 |
| 3 | 5MHz |  |  | QPSK | 6 | Note 2 |
| 4 | 5MHz |  |  | 16QAM | 5 | Note 2 |
| 5 | 10MHz |  |  | QPSK | 1 | 0 |
| 6 | 10MHz |  |  | QPSK | 12 | 0 |
| 7 | 10MHz |  |  | QPSK | 36 | 0 |
| 8 | 10MHz |  |  | QPSK | 24 | 13 |
| 9 | 10MHz |  |  | 16QAM | 5 | 13 |
| 10 | 10MHz |  |  | QPSK | 36 | 13 |
| 11 | 10MHz |  |  | QPSK | 12 | 37 |
| 12 | 10MHz |  |  | QPSK | 1 | 49 |
| 13 | 15MHz |  |  | QPSK | 1 | 0 |
| 14 | 15MHz |  |  | QPSK | 16 | 0 |
| 15 | 15MHz |  |  | QPSK | 36 | 0 |
| 16 | 15MHz |  |  | QPSK | 36 | 19 |
| 17 | 15MHz |  |  | QPSK | 18 | 56 |
| 18 | 15MHz |  |  | QPSK | 1 | 74 |
| 19 | 20MHz |  |  | QPSK | 1 | 0 |
| 20 | 20MHz |  |  | QPSK | 18 | 0 |
| 21 | 20MHz |  |  | QPSK | 36 | 0 |
| 22 | 20MHz |  |  | QPSK | 36 | 25 |
| 23 | 20MHz |  |  | QPSK | 25 | 75 |
| 24 | 20MHz |  |  | QPSK | 1 | 99 |
| Note 1: Test Channel Bandwidths are checked separately for E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. |  |  |  |  |  |  |

Table 6.6.2.2E.4.1-3: Test Configuration Table (network signalled value "NS_06")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Lowest, 5MHz, 10MHz, Highest |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |  |
| Ch BW | Mod'n | RB allocation |  | Mod'n | RB allocation |  |
|  |  | FDD | TDD |  | FDD and HD-FDD | TDD |
| 1.4MHz | N/A for Additional Spectrum Emission Mask testing. |  |  | QPSK | 6 | NA |
| 1.4MHz |  |  |  | QPSK | 5 |  |
| 1.4MHz |  |  |  | 16QAM | 5 |  |
| 3MHz |  |  |  | QPSK | 15 |  |
| 3MHz |  |  |  | QPSK | 4 |  |
| 3MHz |  |  |  | 16QAM | 4 |  |
| 5MHz |  |  |  | QPSK | 25 |  |
| 5MHz |  |  |  | QPSK | 8 |  |
| 5MHz |  |  |  | 16QAM | 5 |  |
| 10MHz |  |  |  | QPSK | 36 |  |
| 10MHz |  |  |  | QPSK | 12 |  |
| 10MHz |  |  |  | 16QAM | 5 |  |
| 15MHz |  |  |  | QPSK | 36 |  |
| 15MHz |  |  |  | QPSK | 16 |  |
| 15MHz |  |  |  | 16QAM | 5 |  |
| 20MHz |  |  |  | QPSK | 36 |  |
| 20MHz |  |  |  | QPSK | 18 |  |
| 20MHz |  |  |  | 16QAM | 5 |  |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. |  |  |  |  |  |  |

Table 6.6.2.2E.4.1-4: Test Configuration Table (network signalled value "NS_07")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  |  |  | Mid range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 10MHz |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Test Number | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD and HD-FDD | RBstart |
| 1 | 10MHz | N/A for Additional Spectrum Emission Mask testing. |  | QPSK | 1 | 0 |
| 2 | 10MHz |  |  | QPSK | 8 | 0 |
| 3 | 10MHz |  |  | QPSK | 6 | 13 |
| 4 | 10MHz |  |  | QPSK | 20 | 13 |
| 5 | 10MHz |  |  | QPSK | 12 | 13 |
| 6 | 10MHz |  |  | QPSK | 16 | 19 |
| 7 | 10MHz |  |  | QPSK | 12 | 19 |
|  | 10MHz |  |  | 16QAM | 5 | 19 |
| 10 | 10MHz |  |  | QPSK | 30 | 19 |
| 12 | 10MHz |  |  | QPSK | 6 | 43 |
| 13 | 10MHz |  |  | QPSK | 2 | 48 |
| 14 | 10MHz |  |  | QPSK | 36 | 0 |
| 15 | 10MHz |  |  | QPSK | 12 | 0 |

6.6.2.2E.4.2	Test procedure

Same test procedure as in clause 6.2.3.4.2 with following exception for HD-FDD:

-	In step 3, slots with transient periods are not under test. Half-duplex guard sub frame are not under test.

6.6.2.2E.4.3	Message contents

Same message contents as in clause 6.6.2.2.4.3.

##### 6.6.2.2E.5	Test requirements

6.6.2.2E.5.1	Test requirements (network signalled value "NS_03"”)

When "NS_03” is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.4.5-1 and 6.2.4.5-2 as appropriate,

and

-	the power of any UE emission shall fulfil requirements in Table 6.6.2.2E.5.1-1 or 6.6.2.2E.5.1-2, as applicable.

Table 6.6.2.2E.5.1-1: Additional requirements (network signalled value "NS_03”), E-UTRA bands ≤ 3GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.5 | -11.5 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 1-2.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 |  |  |  |  | 1 MHz |
| 6-10 |  |  | -23.5 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.5 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.5 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_03 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

Table 6.6.2.2E.5.1-2: Additional requirements (network signalled value "NS_03"), 3GHz < E-UTRA bands ≤ 4.2GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.2 | -11.2 | -13.2 | -16.2 | -18.2 | -19.2 | 30 kHz |
| 1-2.5 | -11.2 | -11.2 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 2.5-2.8 | -23.2 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.2 |  |  |  |  | 1 MHz |
| 6-10 |  |  | -23.2 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.2 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.2 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.2 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_03 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

NOTE:	(only for emission measurement) As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

6.6.2.2E.5.2	Test requirements (network signalled value "NS_04")

When "NS_04" is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.4.5-3 as appropriate,

and

-	the power of any UE emission shall fulfil requirements in Table 6.6.2.2E.5.2-1 or Table.6.6.2.2E.5.2-2, as applicable.

Table 6.6.2.2E.5.2-1: Additional requirements (network signalled value "NS_04"), E-UTRA bands ≤ 3GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.5 | -11.5 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 1-2.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 | -23.5 | -23.5 | -23.5 | -23.5 | 1 MHz |
| 6-10 |  |  |  |  |  |  | 1 MHz |
| 10-15 |  |  |  |  |  |  | 1 MHz |
| 15-20 |  |  |  |  |  |  | 1 MHz |
| 20-25 |  |  |  |  |  |  | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_04 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

Table 6.6.2.2E.5.2-2: Additional requirements (network signalled value "NS_04"), 3GHz < E-UTRA bands ≤ 4.2GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.2 | -11.2 | -13.2 | -16.2 | -18.2 | -19.2 | 30 kHz |
| 1-2.5 | -11.2 | -11.2 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 2.5-2.8 | -23.2 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.2 | -23.2 | -23.2 | -23.2 | -23.2 | 1 MHz |
| 6-10 |  |  |  |  |  |  | 1 MHz |
| 10-15 |  |  |  |  |  |  | 1 MHz |
| 15-20 |  |  |  |  |  |  | 1 MHz |
| 20-25 |  |  |  |  |  |  | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_04 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

NOTE:	(only for emission measurement) As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

Test requirements (network signalled value "NS_06" or "NS_07")

When "NS_06" or "NS_07" is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.4.5-5, 6.2.4.5-6 and 6.2.4.5-7 as appropriate l,

and

-	the power of any UE emission shall fulfil requirements in Table 6.6.2.2E.5.3-1 or Table.6.6.2.2E.5.3-2, as applicable.

Table 6.6.2.2E.5.3-1: Additional requirements (network signalled value "NS_06" or "NS_07"), E-UTRA bands ≤ 3GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | Measurement bandwidth |
| 0-0.1 | -11.5 | -11.5 | -13.5 | -16.5 | 30 kHz |
| 0.1-1 | -11.5 | -11.5 | -11.5 | -11.5 | 100 kHz |
| 1-2.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 |  |  | 1 MHz |
| 6-10 |  |  | -23.5 |  | 1 MHz |
| 10-15 |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.085 MHz. The first and last measurement position with a 100 kHz filter is at ΔfOOB equals to 0.15 MHz and 0.95 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel. Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_06 and NS_07 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |

Table 6.6.2.2E.5.3-2: Additional requirements (network signalled value "NS_06" or "NS_07"), 3GHz < E-UTRA bands ≤ 4.2GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | Measurement bandwidth |
| 0-0.1 | -11.2 | -11.2 | -13.2 | -16.2 | 30 kHz |
| 0.1-1 | -11.2 | -11.2 | -11.2 | -11.2 | 100 kHz |
| 1-2.5 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 2.5-2.8 | -23.2 |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  | 1 MHz |
| 5-6 |  | -23.2 |  |  | 1 MHz |
| 6-10 |  |  | -23.2 |  | 1 MHz |
| 10-15 |  |  |  | -23.2 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.085 MHz. The first and last measurement position with a 100 kHz filter is at ΔfOOB equals to 0.15 MHz and 0.95 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_06 and NS_07 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |

NOTE:	(only for emission measurement) As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

#### 6.6.2.2EA	Additional Spectrum Emission Mask for UE category M1

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- NS_04 (B41): Test configuration table missing due to minimum requirements for A-MPR in 36.101 are incomplete.

##### 6.6.2.2EA.1	Test purpose

Same test purpose as in clause 6.6.2.2.1.

##### 6.6.2.2EA.2	Test applicability

This test case applies to all types of E-UTRA UE release 13 and forward of UE category M1.

##### 6.6.2.2EA.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.6.2.2.3

##### 6.6.2.2EA.4	Test description

6.6.2.2EA.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in sub-clause 5.2E. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Tables 6.6.2.2EA.4.1-1 through 6.6.2.2EA.4.1-4. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2 respectively. Configurations of PDSCH and MPDCCH before measurement are specified in Annex C.2.

Table 6.6.2.2EA.4.1-1: Test Configuration Table (network signalled value "NS_03")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Lowest, 5MHz, 10MHz, Highest |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| Low range, Mid range |  |  |  |  |  |
| 1 | 1.4MHz |  | QPSK | 2 | 0 |
| 2 | 1.4MHz |  | QPSK | 5 | 0 |
| 3 | 1.4MHz |  | QPSK | 6 | 0 |
| 4 | 1.4MHz |  | 16QAM | 2 | 0 |
| 5 | 1.4MHz |  | 16QAM | 5 | 0 |
| 6 | 3MHz |  | QPSK | 2 | 0 |
| 7 | 3MHz |  | QPSK | 5 | 0 |
| 8 | 3MHz |  | QPSK | 6 | 0 |
| 9 | 3MHz |  | 16QAM | 2 | 0 |
| 10 | 3MHz |  | 16QAM | 5 | 0 |
| 11 | 5MHz |  | QPSK | 6 | 0 |
| 12 (Note 4) | 5MHz |  | 16QAM | 1 | 0 |
| 13 | 5MHz |  | 16QAM | 3 | 0 |
| 14 | 5MHz |  | 16QAM | 5 | 0 |
| 15 (Note 4) | 10MHz |  | QPSK | 4 | 0 |
| 16 | 10MHz |  | QPSK | 6 | 0 |
| 17 (Note 4) | 10MHz |  | 16QAM | 3 | 0 |
| 18 | 10MHz |  | 16QAM | 5 | 0 |
| 19 | 15MHz |  | QPSK | 6 | 0 |
| 20 | 15MHz |  | 16QAM | 5 | 0 |
| 21 | 20MHz |  | 16QAM | 5 | 0 |
| High range |  |  |  |  |  |
| 1 | 1.4MHz |  | QPSK | 2 | 0 |
| 2 | 1.4MHz |  | QPSK | 5 | 0 |
| 3 | 1.4MHz |  | QPSK | 6 | 0 |
| 4 | 1.4MHz |  | 16QAM | 2 | 0 |
| 5 | 1.4MHz |  | 16QAM | 5 | 0 |
| 6 | 3MHz |  | QPSK | 2 | 1 |
| 7 | 3MHz |  | QPSK | 5 | 1 |
| 8 | 3MHz |  | QPSK | 6 | 1 |
| 9 | 3MHz |  | 16QAM | 2 | 1 |
| 10 | 3MHz |  | 16QAM | 5 | 1 |
| 11 | 5MHz |  | QPSK | 6 | 3 |
| 12 (Note 4) | 5MHz |  | 16QAM | 1 | 3 |
| 13 | 5MHz |  | 16QAM | 3 | 3 |
| 14 | 5MHz |  | 16QAM | 5 | 3 |
| 15 (Note 4) | 10MHz |  | QPSK | 4 | 7 |
| 16 | 10MHz |  | QPSK | 6 | 7 |
| 17 (Note 4) | 10MHz |  | 16QAM | 3 | 7 |
| 18 | 10MHz |  | 16QAM | 5 | 7 |
| 19 | 15MHz |  | QPSK | 6 | 11 |
| 20 | 15MHz |  | 16QAM | 5 | 11 |
| 21 | 20MHz |  | 16QAM | 6 | 15 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4 Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 - RB allocation) of the narrowband. Note 4: Only for power class 3 UE |  |  |  |  |  |

Table 6.6.2.2EA.4.1-1a: Test Configuration Table, subPRB allocation (network signalled value "NS_03")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low Range, Mid range, High Range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Lowest, 5MHz, 10MHz, Highest |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| Low range, Mid range |  |  |  |  |  |
| 1 | 1.4MHz |  | π/2-BPSK | ¼ | 0 |
| 2 | 1.4MHz |  | QPSK | ½ | 0 |
| 3 | 3MHz |  | π/2-BPSK | ¼ | 0 |
| 4 | 3MHz |  | QPSK | ½ | 0 |
| 5 | 5MHz |  | π/2-BPSK | ¼ | 0 |
| 6 | 5MHz |  | QPSK | ½ | 0 |
| 7 | 10MHz |  | π/2-BPSK | ¼ | 0 |
| 8 | 10MHz |  | QPSK | ½ | 0 |
| 9 | 15MHz |  | π/2-BPSK | ¼ | 0 |
| 10 | 15MHz |  | QPSK | ½ | 0 |
| 11 | 20MHz |  | π/2-BPSK | ¼ | 0 |
| 12 | 20MHz |  | QPSK | ½ | 0 |
| High range |  |  |  |  |  |
| 13 | 1.4MHz |  | π/2-BPSK | ¼ | 0 |
| 14 | 1.4MHz |  | QPSK | ½ | 0 |
| 15 | 3MHz |  | π/2-BPSK | ¼ | 1 |
| 16 | 3MHz |  | QPSK | ½ | 1 |
| 17 | 5MHz |  | π/2-BPSK | ¼ | 3 |
| 18 | 5MHz |  | QPSK | ½ | 3 |
| 19 | 10MHz |  | π/2-BPSK | ¼ | 7 |
| 20 | 10MHz |  | QPSK | ½ | 7 |
| 21 | 15MHz |  | π/2-BPSK | ¼ | 11 |
| 22 | 15MHz |  | QPSK | ½ | 11 |
| 23 | 20MHz |  | π/2-BPSK | ¼ | 15 |
| 24 | 20MHz |  | QPSK | ½ | 15 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4. Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: For Low range, Mid range, the SCstart shall be 0. For High range, SCstart shall be SC# (47 – (RB allocation x 12)) of the narrowband. Note 4: This table applies only to UE supporting subPRB allocation. |  |  |  |  |  |

Table 6.6.2.2EA.4.1-2: Test Configuration Table (network signalled value "NS_04")

TBD

Table 6.6.2.2EA.4.1-2a: Test Configuration Table, subPRB allocation (network signalled value "NS_04")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low Range, Mid range, High Range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 5MHz, 10MHz, 15MHz, 20MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| Low range, Mid range |  |  |  |  |  |
| 1 | 5MHz |  | π/2-BPSK | ¼ | 0 |
| 2 | 5MHz |  | QPSK | ½ | 0 |
| 3 | 10MHz |  | π/2-BPSK | ¼ | 0 |
| 4 | 10MHz |  | QPSK | ½ | 0 |
| 5 | 15MHz |  | π/2-BPSK | ¼ | 0 |
| 6 | 15MHz |  | QPSK | ½ | 0 |
| 7 | 20MHz |  | π/2-BPSK | ¼ | 0 |
| 8 | 20MHz |  | QPSK | ½ | 0 |
| High range |  |  |  |  |  |
| 9 | 5MHz |  | π/2-BPSK | ¼ | 3 |
| 10 | 5MHz |  | QPSK | ½ | 3 |
| 11 | 10MHz |  | π/2-BPSK | ¼ | 7 |
| 12 | 10MHz |  | QPSK | ½ | 7 |
| 13 | 15MHz |  | π/2-BPSK | ¼ | 11 |
| 14 | 15MHz |  | QPSK | ½ | 11 |
| 15 | 20MHz |  | π/2-BPSK | ¼ | 15 |
| 16 | 20MHz |  | QPSK | ½ | 15 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4 Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: For Low range, Mid range, the SCstart shall be 0. For High range, SCstart shall be SC# (47 – (RB allocation x 12)) of the narrowband. Note 4: This table applies only to UE supporting subPRB allocation. |  |  |  |  |  |

Table 6.6.2.2EA.4.1-3: Test Configuration Table (network signalled value "NS_06")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Lowest, 5MHz, 10MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| Low range, Mid range |  |  |  |  |  |
| 1 | 1.4MHz |  | QPSK | 2 | 0 |
| 2 | 1.4MHz |  | QPSK | 5 | 0 |
| 3 | 1.4MHz |  | QPSK | 6 | 0 |
| 4 | 1.4MHz |  | 16QAM | 2 | 0 |
| 5 | 1.4MHz |  | 16QAM | 5 | 0 |
| 6 | 3MHz |  | QPSK | 2 | 0 |
| 7 | 3MHz |  | QPSK | 5 | 0 |
| 8 | 3MHz |  | QPSK | 6 | 0 |
| 9 | 3MHz |  | 16QAM | 2 | 0 |
| 10 | 3MHz |  | 16QAM | 5 | 0 |
| 11 | 5MHz |  | QPSK | 6 | 0 |
| 12 (Note 4) | 5MHz |  | 16QAM | 1 | 0 |
| 13 | 5MHz |  | 16QAM | 3 | 0 |
| 14 | 5MHz |  | 16QAM | 5 | 0 |
| 15 (Note 4) | 10MHz |  | QPSK | 4 | 0 |
| 16 | 10MHz |  | QPSK | 6 | 0 |
| 17 (Note 4) | 10MHz |  | 16QAM | 3 | 0 |
| 18 | 10MHz |  | 16QAM | 5 | 0 |
| High range |  |  |  |  |  |
| 1 | 1.4MHz |  | QPSK | 2 | 0 |
| 2 | 1.4MHz |  | QPSK | 5 | 0 |
| 3 | 1.4MHz |  | QPSK | 6 | 0 |
| 4 | 1.4MHz |  | 16QAM | 2 | 0 |
| 5 | 1.4MHz |  | 16QAM | 5 | 0 |
| 6 | 3MHz |  | QPSK | 2 | 1 |
| 7 | 3MHz |  | QPSK | 5 | 1 |
| 8 | 3MHz |  | QPSK | 6 | 1 |
| 9 | 3MHz |  | 16QAM | 2 | 1 |
| 10 | 3MHz |  | 16QAM | 5 | 1 |
| 11 | 5MHz |  | QPSK | 6 | 3 |
| 12 (Note 4) | 5MHz |  | 16QAM | 1 | 3 |
| 13 | 5MHz |  | 16QAM | 3 | 3 |
| 14 | 5MHz |  | 16QAM | 5 | 3 |
| 15 (Note 4) | 10MHz |  | QPSK | 4 | 7 |
| 16 | 10MHz |  | QPSK | 6 | 7 |
| 17 (Note 4) | 10MHz |  | 16QAM | 3 | 7 |
| 18 | 10MHz |  | 16QAM | 5 | 7 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4 Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 - RB allocation) of the narrowband. Note 4: Only for power class 3 UE |  |  |  |  |  |

Table 6.6.2.2EA.4.1-4: Test Configuration Table (network signalled value "NS_07")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 10MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| Low range, Mid range |  |  |  |  |  |
| 1 | 10MHz |  | QPSK | 1 | 0 |
| 2 | 10MHz |  | QPSK | 4 | 0 |
| 3 | 10MHz |  | QPSK | 6 | 0 |
| 4 | 10MHz |  | 16QAM | 5 | 0 |
| High range |  |  |  |  |  |
| 1 | 10MHz |  | QPSK | 1 | 7 |
| 2 | 10MHz |  | QPSK | 4 | 7 |
| 3 | 10MHz |  | QPSK | 6 | 7 |
| 4 | 10MHz |  | 16QAM | 5 | 7 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4 Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 - RB allocation) of the narrowband. |  |  |  |  |  |

Table 6.6.2.2EA.4.1-4a: Test Configuration Table, subPRB allocation (network signalled value "NS_07")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 10MHz |  |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |  |
| Configuration ID | Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |  |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) | SCstart |
| Low range, Mid range |  |  |  |  |  |  |
| 1 | 10MHz |  | π/2-BPSK | ¼ | 0 | 0 |
| High range |  |  |  |  |  |  |
| 1 | 10MHz |  | π/2-BPSK | ¼ | 0 | 8 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4. Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. |  |  |  |  |  |  |

1.	Connect SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The DL Reference Measurement channels are set according to Tables 6.6.2.2EA.4.1-1 to 6.6.2.2EA.4.1-4.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [7] clause 5.2A.2AA. Message contents are defined in clause 6.6.2.2EA.4.3.

6.6.2.2EA.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via MPDCCH DCI format 6-0A for C_RNTI to schedule the UL RMC according to applicable table from Table 6.6.2.2EA.4.1-1 to 6.6.2.2.4EA.1-4. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

3.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

4.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Table 6.2.4EA.5-1 to 6.2.4EA.5-22 as appropriate. The period of the measurement shall be at least one sub-frame (1ms). For TDD slots with transient periods are not under test. For HD-FDD slots with transient periods and Half-duplex guard subframe are not under test

5.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to Table 6.6.2.2EA.5.1-1, 6.6.2.2EA.5.2-1 and, 6.6.2.2.5EA.3-1 as applicable and using a rms detector. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.	For UE supporting subPRB allocation, repeat step 1-5 with UL RMC according to applicable Table from Table 6.6.2.2EA.4.1-1a to 6.6.2.2.4EA.1-4a

6.6.2.2EA.4.3	Message contents

Message contents are same as in clause 6.2.4EA.4.3.

##### 6.6.2.2EA.5	Test requirements

6.6.2.2EA.5.1	Test requirements (network signalled value "NS_03"”)

When "NS_03” is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.4EA.5-1 and 6.2.4EA.5-2 as appropriate,

and

-	the power of any UE emission shall fulfil requirements in Table 6.6.2.2EA.5.1-1, as applicable.

Table 6.6.2.2EA.5.1-1: Additional requirements (network signalled value "NS_03”)

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.5 | -11.5 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 1-2.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 |  |  |  |  | 1 MHz |
| 6-10 |  |  | -23.5 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.5 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.5 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_03 as defined in TS 36.101 [2] subclause 6.2.4 E Table 6.2.4 E -1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

NOTE:	(only for emission measurement) As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

6.6.2.2EA.5.2	Test requirements (network signalled value "NS_04")

When "NS_04" is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.4EA.5-3 as appropriate,

and

-	the power of any UE emission shall fulfil requirements in Table 6.6.2.2EA.5.2-1, as applicable.

Table 6.6.2.2EA.5.2-1: Additional requirements (network signalled value "NS_04")

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.5 | -11.5 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 1-2.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 | -23.5 | -23.5 | -23.5 | -23.5 | 1 MHz |
| 6-10 |  |  |  |  |  |  | 1 MHz |
| 10-15 |  |  |  |  |  |  | 1 MHz |
| 15-20 |  |  |  |  |  |  | 1 MHz |
| 20-25 |  |  |  |  |  |  | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_04 as defined in TS 36.101 [2] subclause 6.2.4 E Table 6.2.4 E -1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

NOTE:	(only for emission measurement) As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

Test requirements (network signalled value "NS_06" or "NS_07")

When "NS_06" or "NS_07" is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.4EA.5-5, 6.2.4EA.5-6 and 6.2.4EA.5-7 as appropriate ,

and

-	the power of any UE emission shall fulfil requirements in Table 6.6.2.2EA.5.3-1, as applicable.

Table 6.6.2.2EA.5.3-1: Additional requirements (network signalled value "NS_06" or "NS_07")

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | Measurement bandwidth |
| 0-0.1 | -11.5 | -11.5 | -13.5 | -16.5 | 30 kHz |
| 0.1-1 | -11.5 | -11.5 | -11.5 | -11.5 | 100 kHz |
| 1-2.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 |  |  | 1 MHz |
| 6-10 |  |  | -23.5 |  | 1 MHz |
| 10-15 |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.085 MHz. The first and last measurement position with a 100 kHz filter is at ΔfOOB equals to 0.15 MHz and 0.95 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_06 and NS_07 as defined in TS 36.101 [2] subclause 6.2.4 E Table 6.2.4 E -1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |

NOTE:	(only for emission measurement) As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

#### 6.6.2.2EB	Additional Spectrum Emission Mask for UE category 1bis

##### 6.6.2.2EB.1	Test purpose

Same test purpose as in clause 6.6.2.2.1.

##### 6.6.2.2EB.2	Test applicability

This test case applies to all types of E-UTRA UE release 13 and forward of UE category 1bis.

##### 6.6.2.2EB.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.6.2.2.3

##### 6.6.2.2EB.4	Test description

6.6.2.2EB.4.1	Initial conditions

Same initial conditions as in clause 6.6.2.2.4.1 with following exceptions:

-	Connect SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.3 using only main UE Tx/Rx antenna.

6.6.2.2EB.4.2	Test procedure

Same test procedure as in clause 6.2.3.4.2.

6.6.2.2EB.4.3	Message contents

Same message contents as in clause 6.6.2.2.4.3.

##### 6.6.2.2EB.5	Test requirements

6.6.2.2EB.5.1	Test requirements (network signalled value "NS_03"”)

When "NS_03” is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.4.5-1 and 6.2.4.5-2 as appropriate,

and

-	the power of any UE emission shall fulfil requirements in Table 6.6.2.2EB.5.1-1 or 6.6.2.2EB.5.1-2, as applicable.

Table 6.6.2.2EB.5.1-1: Additional requirements (network signalled value "NS_03”), E-UTRA bands ≤ 3GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.5 | -11.5 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 1-2.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 |  |  |  |  | 1 MHz |
| 6-10 |  |  | -23.5 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.5 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.5 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_03 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

Table 6.6.2.2EB.5.1-2: Additional requirements (network signalled value "NS_03"), 3GHz < E-UTRA bands ≤ 4.2GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.2 | -11.2 | -13.2 | -16.2 | -18.2 | -19.2 | 30 kHz |
| 1-2.5 | -11.2 | -11.2 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 2.5-2.8 | -23.2 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.2 |  |  |  |  | 1 MHz |
| 6-10 |  |  | -23.2 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.2 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.2 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.2 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_03 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

NOTE:	(only for emission measurement) As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

6.6.2.2EB.5.2	Test requirements (network signalled value "NS_04")

When "NS_04" is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.4.5-3 as appropriate,

and

-	the power of any UE emission shall fulfil requirements in Table 6.6.2.2EB.5.2-1 or Table.6.6.2.2EB.5.2-2, as applicable.

Table 6.6.2.2EB.5.2-1: Additional requirements (network signalled value "NS_04"), E-UTRA bands ≤ 3GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.5 | -11.5 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 1-2.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 | -23.5 | -23.5 | -23.5 | -23.5 | 1 MHz |
| 6-10 |  |  |  |  |  |  | 1 MHz |
| 10-15 |  |  |  |  |  |  | 1 MHz |
| 15-20 |  |  |  |  |  |  | 1 MHz |
| 20-25 |  |  |  |  |  |  | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_04 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

Table 6.6.2.2EB.5.2-2: Additional requirements (network signalled value "NS_04"), 3GHz < E-UTRA bands ≤ 4.2GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.2 | -11.2 | -13.2 | -16.2 | -18.2 | -19.2 | 30 kHz |
| 1-2.5 | -11.2 | -11.2 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 2.5-2.8 | -23.2 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.2 | -23.2 | -23.2 | -23.2 | -23.2 | 1 MHz |
| 6-10 |  |  |  |  |  |  | 1 MHz |
| 10-15 |  |  |  |  |  |  | 1 MHz |
| 15-20 |  |  |  |  |  |  | 1 MHz |
| 20-25 |  |  |  |  |  |  | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_04 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

NOTE:	(only for emission measurement) As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

Test requirements (network signalled value "NS_06" or "NS_07")

When "NS_06" or "NS_07" is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.4.5-5, 6.2.4.5-6 and 6.2.4.5-7 as appropriate l,

and

-	the power of any UE emission shall fulfil requirements in Table 6.6.2.2EB.5.3-1 or Table.6.6.2.2EB.5.3-2, as applicable.

Table 6.6.2.2EB.5.3-1: Additional requirements (network signalled value "NS_06" or "NS_07"), E-UTRA bands ≤ 3GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | Measurement bandwidth |
| 0-0.1 | -11.5 | -11.5 | -13.5 | -16.5 | 30 kHz |
| 0.1-1 | -11.5 | -11.5 | -11.5 | -11.5 | 100 kHz |
| 1-2.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 |  |  | 1 MHz |
| 6-10 |  |  | -23.5 |  | 1 MHz |
| 10-15 |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.085 MHz. The first and last measurement position with a 100 kHz filter is at ΔfOOB equals to 0.15 MHz and 0.95 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel. Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_06 and NS_07 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |

Table 6.6.2.2EB.5.3-2: Additional requirements (network signalled value "NS_06" or "NS_07"), 3GHz < E-UTRA bands ≤ 4.2GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | Measurement bandwidth |
| 0-0.1 | -11.2 | -11.2 | -13.2 | -16.2 | 30 kHz |
| 0.1-1 | -11.2 | -11.2 | -11.2 | -11.2 | 100 kHz |
| 1-2.5 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 2.5-2.8 | -23.2 |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  | 1 MHz |
| 5-6 |  | -23.2 |  |  | 1 MHz |
| 6-10 |  |  | -23.2 |  | 1 MHz |
| 10-15 |  |  |  | -23.2 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.085 MHz. The first and last measurement position with a 100 kHz filter is at ΔfOOB equals to 0.15 MHz and 0.95 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_06 and NS_07 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |

NOTE:	(only for emission measurement) As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

#### 6.6.2.2EC	Additional Spectrum Emission Mask for UE category M2

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- NS_03 (B2, B4), NS_04 (B41) and NS_07 (B13): Test configuration table missing due to minimum requirements for A-MPR in 36.101 are incomplete.

##### 6.6.2.2EC.1	Test purpose

Same test purpose as in clause 6.6.2.2.1.

##### 6.6.2.2EC.2	Test applicability

This test case applies to all types of E-UTRA UE release 14 and forward of UE category M2.

##### 6.6.2.2EC.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.6.2.2.3

##### 6.6.2.2EC.4	Test description

6.6.2.2EC.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in sub-clause 5.2E. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Tables 6.6.2.2EC.4.1-1 through 6.6.2.2EC.4.1-5. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2 respectively. Configurations of PDSCH and MPDCCH before measurement are specified in Annex C.2.

Table 6.6.2.2EC.4.1-1: Test Configuration Table (network signalled value "NS_03")

TBD

Table 6.6.2.2EC.4.1-2: Test Configuration Table (network signalled value "NS_04")

TBD

Table 6.6.2.2EC.4.1-3: Test Configuration Table (network signalled value "NS_06")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Lowest, 5MHz, 10MHz, Highest |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| Low range, Mid range |  |  |  |  |  |
| 1 | 1.4MHz |  | QPSK | 2 | 0 |
| 2 | 1.4MHz |  | QPSK | 5 | 0 |
| 3 | 1.4MHz |  | QPSK | 6 | 0 |
| 4 | 1.4MHz |  | 16QAM | 2 | 0 |
| 5 | 1.4MHz |  | 16QAM | 6 | 0 |
| 6 (Note 4) | 5MHz |  | QPSK | 1 | 0 |
| 7 (Note 5) | 5MHz |  | QPSK | 3 | 0 |
| 8 | 5MHz |  | QPSK | 24 | 0 |
| 9 (Note 4) | 5MHz |  | 16QAM | 1 | 0 |
| 10 | 5MHz |  | 16QAM | 3 | 0 |
| 11 (Note 5) | 5MHz |  | 16QAM | 5 | 0 |
| 12 | 5MHz |  | 16QAM | 24 | 0 |
| 13 (Note 4) | 10MHz |  | QPSK | 4 | 0 |
| 14 (Note 5) | 10MHz |  | QPSK | 5 | 0 |
| 15 | 10MHz |  | QPSK | 24 | 0 |
| 16 (Note 4) | 10MHz |  | 16QAM | 3 | 0 |
| 17 | 10MHz |  | 16QAM | 5 | 0 |
| 18 | 10MHz |  | 16QAM | 24 | 0 |
| High range |  |  |  |  |  |
| 1 | 1.4MHz |  | QPSK | 2 | 0 |
| 2 | 1.4MHz |  | QPSK | 5 | 0 |
| 3 | 1.4MHz |  | QPSK | 6 | 0 |
| 4 | 1.4MHz |  | 16QAM | 2 | 0 |
| 5 | 1.4MHz |  | 16QAM | 6 | 0 |
| 6 (Note 4) | 5MHz |  | QPSK | 1 | 3 |
| 7 (Note 5) | 5MHz |  | QPSK | 3 | 3 |
| 8 | 5MHz |  | QPSK | 24 | 0 |
| 9 (Note 4) | 5MHz |  | 16QAM | 1 | 3 |
| 10 | 5MHz |  | 16QAM | 3 | 3 |
| 11 (Note 5) | 5MHz |  | 16QAM | 5 | 3 |
| 12 | 5MHz |  | 16QAM | 24 | 0 |
| 13 (Note 4) | 10MHz |  | QPSK | 4 | 7 |
| 14 (Note 5) | 10MHz |  | QPSK | 5 | 7 |
| 15 | 10MHz |  | QPSK | 24 | 4 |
| 16 (Note 4) | 10MHz |  | 16QAM | 3 | 7 |
| 17 | 10MHz |  | 16QAM | 5 | 7 |
| 18 | 10MHz |  | 16QAM | 24 | 4 |
| Note 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.7. Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 – (RB allocation mod 6)) of the narrowband. Note 4: Only for power class 3 UE Note 5: Only for power class 5 and power class 6 UE |  |  |  |  |  |

Table 6.6.2.2EC.4.1-4: Test Configuration Table (network signalled value "NS_07")

TBD

Table 6.6.2.2EC.4.1-5: Test Configuration Table (network signalled value "NS_35")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Lowest, 5MHz, 10MHz, Highest |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| Low range, Mid range |  |  |  |  |  |
| 1 (Note 4) | 5MHz |  | QPSK | 1 | 0 |
| 2 (Note 5) | 5MHz |  | QPSK | 3 | 0 |
| 3 | 5MHz |  | QPSK | 24 | 0 |
| 4 (Note 4) | 5MHz |  | 16QAM | 1 | 0 |
| 5 | 5MHz |  | 16QAM | 3 | 0 |
| 6 (Note 5) | 5MHz |  | 16QAM | 5 | 0 |
| 7 | 5MHz |  | 16QAM | 24 | 0 |
| 8 | 10MHz |  | QPSK | 4 | 0 |
| 9 (Note 5) | 10MHz |  | QPSK | 5 | 0 |
| 10 | 10MHz |  | QPSK | 24 | 0 |
| 11 (Note 4) | 10MHz |  | 16QAM | 3 | 0 |
| 12 | 10MHz |  | 16QAM | 5 | 0 |
| 13 | 10MHz |  | 16QAM | 24 | 0 |
| 14 | 20MHz |  | QPSK | 18 | 0 |
| 15 | 20MHz |  | QPSK | 24 | 0 |
| 16 | 20MHz |  | 16QAM | 18 | 0 |
| 17 | 20MHz |  | 16QAM | 24 | 0 |
| High range |  |  |  |  |  |
| 1 (Note 4) | 5MHz |  | QPSK | 1 | 3 |
| 2 (Note 5) | 5MHz |  | QPSK | 3 | 3 |
| 3 | 5MHz |  | QPSK | 24 | 0 |
| 4 (Note 4) | 5MHz |  | 16QAM | 1 | 3 |
| 5 | 5MHz |  | 16QAM | 3 | 3 |
| 6 (Note 5) | 5MHz |  | 16QAM | 5 | 3 |
| 7 | 5MHz |  | 16QAM | 24 | 0 |
| 8 | 10MHz |  | QPSK | 4 | 7 |
| 9 (Note 5) | 10MHz |  | QPSK | 5 | 7 |
| 10 | 10MHz |  | QPSK | 24 | 4 |
| 11 (Note 4) | 10MHz |  | 16QAM | 3 | 7 |
| 12 | 10MHz |  | 16QAM | 5 | 7 |
| 13 | 10MHz |  | 16QAM | 24 | 4 |
| 14 | 20MHz |  | QPSK | 18 | 13 |
| 15 | 20MHz |  | QPSK | 24 | 12 |
| 16 | 20MHz |  | 16QAM | 18 | 13 |
| 17 | 20MHz |  | 16QAM | 24 | 12 |
| Note 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.7. Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 – (RB allocation mod 6)) of the narrowband. Note 4: Only for power class 3 UE Note 5: Only for power class 5 and power class 6 UE |  |  |  |  |  |

1.	Connect SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The DL Reference Measurement channels are set according to Tables 6.6.2.2EC.4.1-1 to 6.6.2.2EC.4.1-5.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [7] clause 5.2A.2AA. Message contents are defined in clause 6.6.2.2EC.4.3.

6.6.2.2EC.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via MPDCCH DCI format 6-0A for C_RNTI to schedule the UL RMC according to applicable table from Table 6.6.2.2EC.4.1-1 to 6.6.2.2EC.4.1-5. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

3.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

4.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Table 6.2.4EA.5-1 to 6.2.4EA.5-16 as appropriate. The period of the measurement shall be at least one sub-frame (1ms). For HD-FDD slots with transient periods and Half-duplex guard subframe are not under test

5.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to Table 6.6.2.2EC.5.1-1, 6.6.2.2EC.5.2-1, 6.6.2.2EC.5.3-1 and, 6.6.2.2EC.5.4-1 as applicable and using a rms detector. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.6.2.2EC.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the condition CEModeA and with the same exceptions as in clauses 6.2.4.4.3.1 to 6.2.4.4.3.30 for each network signalled value.

Message contents are same as in clause 6.2.4EA.4.3.

##### 6.6.2.2EC.5	Test requirements

6.6.2.2EC.5.1	Test requirements (network signalled value "NS_03”)

When "NS_03” is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.4EC.5-1 as appropriate,

and

-	the power of any UE emission shall fulfil requirements in Table 6.6.2.2EC.5.1-1, as applicable.

Table 6.6.2.2EC.5.1-1: Additional requirements (network signalled value "NS_03”)

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.5 | -11.5 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 1-2.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 |  |  |  |  | 1 MHz |
| 6-10 |  |  | -23.5 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.5 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.5 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_03 as defined in TS 36.101 [2] subclause 6.2.4E Table 6.2.4E-2. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

NOTE:	(only for emission measurement) As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

6.6.2.2EC.5.2	Test requirements (network signalled value "NS_04")

When "NS_04" is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.4EC.5-2 as appropriate,

and

-	the power of any UE emission shall fulfil requirements in Table 6.6.2.2EC.5.2-1, as applicable.

Table 6.6.2.2EC.5.2-1: Additional requirements (network signalled value "NS_04")

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.5 | -11.5 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 1-2.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 | -23.5 | -23.5 | -23.5 | -23.5 | 1 MHz |
| 6-10 |  |  |  |  |  |  | 1 MHz |
| 10-15 |  |  |  |  |  |  | 1 MHz |
| 15-20 |  |  |  |  |  |  | 1 MHz |
| 20-25 |  |  |  |  |  |  | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_04 as defined in TS 36.101 [2] subclause 6.2.4E Table 6.2.4E-2. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

NOTE:	(only for emission measurement) As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

6.6.2.2EC.5.3	Test requirements (network signalled value "NS_06" or "NS_07")

When "NS_06" or "NS_07" is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.4EC.5-4A, 6.2.4EC.5-4B and 6.2.4EC.5-5 as appropriate,

and

-	the power of any UE emission shall fulfil requirements in Table 6.6.2.2EC.5.3-1, as applicable.

Table 6.6.2.2EC.5.3-1: Additional requirements (network signalled value "NS_06" or "NS_07")

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | Measurement bandwidth |
| 0-0.1 | -11.5 | -11.5 | -13.5 | -16.5 | 30 kHz |
| 0.1-1 | -11.5 | -11.5 | -11.5 | -11.5 | 100 kHz |
| 1-2.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 |  |  | 1 MHz |
| 6-10 |  |  | -23.5 |  | 1 MHz |
| 10-15 |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.085 MHz. The first and last measurement position with a 100 kHz filter is at ΔfOOB equals to 0.15 MHz and 0.95 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_06 and NS_07 as defined in TS 36.101 [2] subclause 6.2.4E Table 6.2.4E-2. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |

NOTE:	(only for emission measurement) As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

6.6.2.2EC.5.4	Test requirements (network signalled value "NS_35")

When "NS_35" is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.4EC.5-16 as appropriate,

and

-	the power of any UE emission shall fulfil requirements in Table 6.6.2.2EC.5.4-1, as applicable.

Table 6.6.2.2EC.5.4-1: Additional requirements (network signalled value "NS_35")

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth (unless otherwise stated) |
| 0-0.1 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 0.1-6 | -11.5 | -11.5 | -11.5 | -11.5 | 100 kHz |
| 6-10 | -23.55 | -11.5 | -11.5 | -11.5 | 100 kHz |
| 10-15 |  | -23.55 | -11.5 | -11.5 | 100 kHz |
| 15-20 |  |  | -23.55 | -11.5 | 100 kHz |
| 20-25 |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.085 MHz. The first and last measurement position with a 100 kHz filter is at ΔfOOB equals to 0.15 MHz and 0.95 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_35 as defined in TS 36.101 [2] subclause 6.2.4E Table 6.2.4E-2. Note 5: The measurement bandwidth shall be 1 MHz |  |  |  |  |  |

#### 6.6.2.2G	Additional Spectrum Emission Mask for V2X Communication

##### 6.6.2.2G.0	Minimum Requirements

TS 36.101 [2] clause 6.6.2G

When UE is configured for E-UTRA V2X sidelink transmissions non-concurrent with E-UTRA uplink transmissions for E-UTRA V2X operating bands specified in Table 5.5G-1, the requirements in subclause 6.6.2 apply except for the ACLR requirements for power class 2 V2X UE.

When UE is configured for simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions for inter-band E-UTRA V2X / E-UTRA bands specified in Table 5.5G-2, the requirements in subclause 6.6.2 apply per V2X sidelink transmission and E-UTRA uplink transmission as specified for the corresponding inter-band con-current operation with uplink assigned to two bands.

For intra-band contiguous multi-carrier operation, the general CA spectrum emission mask for CA Bandwidth Class B specified in subclause 6.6.2.1A shall apply for V2X Bandwidth Class B, the general CA spectrum emission mask for CA Bandwidth Class C specified in subclause 6.6.2.1A shall apply for V2X Bandwidth Class C and C1.

TS 36.101 [2] clause 6.2.2.4

The additional spectrum mask in Table 6.6.2.2.4-1 applies for E-UTRA V2X UE within 5 855 MHz to 5 950 MHz according to ETSI EN 302 571. Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When "NS_33" or “NS_34” is indicated in the cell, the power of any V2X UE emission shall not exceed the levels specified in Table 6.6.2.2.4-1.

Table 6.6.2.2.4-1: Additional requirements for 10MHz channel bandwidth

| Spectrum emission limit (dBm EIRP)/ Channel bandwidth |  |  |
| --- | --- | --- |
| ΔfOOB (MHz) | 10 MHz | Measurement bandwidth |
| 0-0.5 | [] | 100 kHz |
| 0.5-5 | [] | 100 kHz |
| 5-10 | [] | 100 kHz |

NOTE 1:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

NOTE 2:	Additional SEM for V2X overrides any other requirements in frequency range 5855-5950MHz.

NOTE 3:	The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I.

The normative reference for this requirement is TS 36.101 [2] clauses 6.6.2G, and 6.6.2.2.4 and 6.2.4G where the A-MPR requirement is specified.

##### 6.6.2.2G.1	Additional Spectrum Emission Mask for V2X Communication / Non-concurrent with E-UTRA uplink transmissions

###### 6.6.2.2G.1.1	Test purpose

To verify that the power of V2X UE emission shall not exceed specified level for the specified channel bandwidth under the deployment scenarios where additional requirements are specified.

###### 6.6.2.2G.1.2	Test applicability

This test case applies to UE which support V2X Sidelink communication and Band 47.

###### 6.6.2.2G.1.3	Minimum conformance requirements

The additional spectrum mask in Table 6.6.2.2G.1.3-1 applies for E-UTRA V2X UE within 5855 MHz to 5950 MHz according to ETSI EN 302 571. Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When "NS_33" or "NS_34" is indicated in the cell, the power of any V2X UE emission shall not exceed the levels specified in Table 6.6.2.2G.1.3-1.

Table 6.6.2.2G.1.3-1: Additional requirements for 10MHz channel bandwidth

| Spectrum emission limit (dBm)/ Channel bandwidth |  |  |
| --- | --- | --- |
| ΔfOOB (MHz) | 10 MHz | Measurement bandwidth |
| 0-0.5 | [-13-12∆fOOBMHz] | 100 kHz |
| 0.5-5 | [-19-169∆fOOBMHz-0.5] | 100 kHz |
| 5-10 | [-27-2∆fOOBMHz-5.0] | 100 kHz |

NOTE 1:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

NOTE 2:	Additional SEM for V2X overrides any other requirements in frequency range 5855-5950MHz.

The normative reference for this requirement is TS 36.101 [2] clause 6.6.2.2.4.

###### 6.6.2.2G.1.4	Test description

6.6.2.2G.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in sub-clause 5.2G. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.2.4G.1.4.1-1 the details of the V2X reference measurement channels (RMCs) are specified in sub-clause A.8.3 and the GNSS configuration in TS 36.508 [7] subclause 4.11.

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 36.508[7] Figure A.92.

2.	The parameter settings for the V2X sidelink transmission over PC5 for both SS and UE are pre-configured according to TS 36.508 [7] subclause 4.10.1 with the exception specified in clause 6.6.2.2G.1.4.3.

3.	The V2X reference measurement channel is set according to Table 6.2.4G.1.4.1-1

4.	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS36.508 [7] Table 4.11.2-2. Geographical area #1 is also pre-configured in the UE.

5.	Propagation conditions are set according to Annex B.0;

6.	Ensure the UE is in State 5A-V2X in Transmit Mode according to TS 36.508 [7] clause 4.5.9.

7.	The GNSS simulator is triggered to start step 1 of Scenario #1 to simulate a location in the centre of Geographical area #1. Wait for the UE to acquire the GNSS signal and start to transmit.

6.6.2.2G.1.4.2	Test procedure

1.	The V2X UE schedules the V2X RMC with transmission power at PUMAX level according to SL-V2X-Preconfiguration which is in line with the test configuration in Table 6.2.4G.1.4.1-1;

2.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in subclass 6.2.4G.1.5 The period of the measurement shall be at least the continuous duration of one sub-frame (1ms).

3.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to table 6.6.2.2G.1.3-1 and using a rms detector. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.6.2.2G.1.4.3	Message contents

Refer to the subclause 6.2.4G.1.4.3

###### 6.6.2.2G.1.5	Test requirements

When "NS_33" or "NS_34" is indicated in the cell,

-	the measured UE mean power in the channel bandwidth, derived in step 2, shall fulfil the requirements described in subclass 6.2.4G.1.5;

And

-	The power of any UE emission shall fulfil requirements in Table 6.6.2.2G.1.5-1 as applicable.

The power of any V2X UE emission shall not exceed the levels specified in Table 6.6.2.2G.1.3-1.

Table 6.6.2.2G.1.5-1: Additional requirements for 10MHz channel bandwidth

| Spectrum emission limit (dBm)/ Channel bandwidth |  |  |
| --- | --- | --- |
| ΔfOOB (MHz) | 10 MHz | Measurement bandwidth |
| 0-0.5 | [-13-12∆fOOBMHz] +2 | 100 kHz |
| 0.5-5 | [-19-169∆fOOBMHz-0.5]+2 | 100 kHz |
| 5-10 | [-27-2∆fOOBMHz-5.0]+2 | 100 kHz |

##### 6.6.2.2G.2	Additional Spectrum Emission Mask for V2X Communication / Sidelink simultaneous with E-UTRA uplink transmissions

###### 6.6.2.2G.2.1	Test purpose

To verify that the power of V2X UE emission shall not exceed specified level for the specified channel bandwidth under the deployment scenarios where additional requirements are specified.

###### 6.6.2.2G.2.2	Test applicability

This test case applies to E-UTRA UE release 14 and forward which support V2X communication and is capable of V2X Sidelink communication simultaneous with E-UTRA uplink transmissions.

###### 6.6.2.2G.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.2.2G.0.

###### 6.6.2.2G.2.4 	Test description

6.6.2.2G.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC bandwidths based on Inter-band con-current V2X specified in sub-clause 5.4.2G. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.2.4G.2.4.1-1 the details of the uplink and V2X reference measurement channels (RMCs) are specified in sub-clause A.2 and A.8.3 respectively. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.93a.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3. Message content exceptions are defined in clause 6.6.2.2G.2.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL and V2X Reference Measurement Channels are set according to subclause 6.2.4G.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in the state 3A-RF-V2X according to TS 36.508 [7] clause 5.2A.2C. Message contents are defined in clause 6.6.2.2G.2.4.3.

6.6.2.2G.2.4.2	Test procedure

1.	SS sends RRC reconfiguration message specified in clause 6.6.2.2G.2.4.3 to enable PSCCH and PSSCH transmission at the PUMAX level.

2.	The SS simultaneously sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC and V2X scheduling information via PDCCH DCI format 5A for SL_V_RNTI to schedule the V2X RMC according to Table 6.6.2.2G.2.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL and V2X RMCs.

3.	SS sends continuously “uplink power control” up commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

4.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in subclass 6.2.4G.1.5. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). On E-UTRA carrier, the TDD slots with transient periods are not under test.

5.	Measure the power of the transmitted signal for E-UTRA uplink with a measurement filter of bandwidths according to table 6.6.2.2G.2.5-1or 6.6.2.2G.2.5-2 and using a rms detector, as applicable. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs. On E-UTRA carrier, the TDD slots with transient periods are not under test.

6.	Measure the power of the transmitted signal for V2X sidelink with a measurement filter of bandwidths according to table 6.6.2.2G.2.5-3 and using a rms detector, as applicable. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.6.2.2G.2.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the following exceptions as defined in sub-clause 6.2.4G.2.4.3

###### 6.6.2.2G.2.5	Test requirements

When "NS_33" or "NS_34" is indicated in the cell,

-	the measured UE mean power in the channel bandwidth, derived in step 4, shall fulfil the requirements described in subclause 6.2.4G.1.5 for Power Class 3 or Power Class 2 UEs;

And

-	The power of any UE emission shall fulfil requirements in Table 6.6.2.2G.2.5-1, 6.6.2.2G.2.5-2 or 6.6.2.2G.2.5-3  as applicable.

The E-UTRA uplink emission derived in step 5 shall fulfil requirements in Table 6.6.2.2G.2.5-1 or Table 6.6.2.2G.2.5-1 as applicable.

Table 6.6.2.2G.2.5-1: General E-UTRA spectrum emission mask, E-UTRA bands ≤ 3GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.5 | -11.5 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 1-2.5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 6-10 |  |  | -23.5 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.5 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.5 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

Table 6.6.2.2G.2.5-2: General E-UTRA spectrum emission mask, 3GHz < E-UTRA bands ≤ 4.2GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.2 | -11.2 | -13.2 | -16.2 | -18.2 | -19.2 | 30 kHz |
| 1-2.5 | -8.2 | -8.2 | -8.2 | -8.2 | -8.2 | -8.2 | 1 MHz |
| 2.5-2.8 | -23.2 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.2 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 6-10 |  |  | -23.2 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.2 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.2 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.2 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

The power of any V2X sidelink emission derived in step 6 shall not exceed the levels specified in Table 6.6.2.2G.2.5-3.

Table 6.6.2.2G.2.5-3: Additional requirements for 10MHz channel bandwidth

| Spectrum emission limit (dBm)/ Channel bandwidth |  |  |
| --- | --- | --- |
| ΔfOOB (MHz) | 10 MHz | Measurement bandwidth |
| 0-0.5 | [-13-12∆fOOBMHz] + 2 | 100 kHz |
| 0.5-5 | [-19-169∆fOOBMHz-0.5]+ 2 | 100 kHz |
| 5-10 | [-27-2∆fOOBMHz-5.0]+ 2 | 100 kHz |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

#### 6.6.2.2_s	Additional Spectrum Emission Mask for subslot/slot TTI

6.6.2.2_s.1	Test purpose

To verify that the power of any UE emission shall not exceed specified level for the specified channel bandwidth under the deployment scenarios where additional requirements are specified.

6.6.2.2_s.2	Test applicability

This test case applies to all types of E-UTRA Power Class 3 UE release 15 and forward that support subslot/slot TTI.

6.6.2.2_s.3	Minimum conformance requirements

6.6.2.2_s.3.1	Minimum requirement (network signalled value "NS_03", “NS_11”, “NS_20” and “NS_21”)

When "NS_03", “NS_11”, “NS_20” or “NS_21” is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.2.2_s.3.1-1.

Table 6.6.2.2_s.3.1-1: Additional requirements (network signalled value "NS_03", “NS_11”, “NS_20” and “NS_21”)

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -10 | -13 | -15 | -18 | -20 | -21 | 30 kHz |
| 1-2.5 | -13 | -13 | -13 | -13 | -13 | -13 | 1 MHz |
| 2.5-2.8 | -25 | -13 | -13 | -13 | -13 | -13 | 1 MHz |
| 2.8-5 |  | -13 | -13 | -13 | -13 | -13 | 1 MHz |
| 5-6 |  | -25 | -13 | -13 | -13 | -13 | 1 MHz |
| 6-10 |  |  | -25 | -13 | -13 | -13 | 1 MHz |
| 10-15 |  |  |  | -25 | -13 | -13 | 1 MHz |
| 15-20 |  |  |  |  | -25 | -13 | 1 MHz |
| 20-25 |  |  |  |  |  | -25 | 1 MHz |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

The normative reference for this requirement is TS 36.101 [2] clause 6.6.2.2.1.

6.6.2.2_s.3.2	Minimum requirement (network signalled value "NS_04")

When "NS_04" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.2.2_s.3.2-2.

Table 6.6.2.2_s.3.2-2: Additional requirements (network signalled value "NS_04")

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -15 | -18 | -20 | -21 | 30 kHz |
| 1-2.5 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.5-2.8 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.8-5 | -10 | -10 | -10 | -10 | 1 MHz |
| 5-6 | -13 | -13 | -13 | -13 | 1 MHz |
| 6-9 | -25 | -13 | -13 | -13 | 1 MHz |
| 9-10 | -25 | -25 | -13 | -13 | 1 MHz |
| 10-13.5 |  | -25 | -13 | -13 | 1 MHz |
| 13.5-15 |  | -25 | -25 | -13 | 1 MHz |
| 15-18 |  |  | -25 | -13 | 1 MHz |
| 18-20 |  |  | -25 | -25 | 1 MHz |
| 20-25 |  |  |  | -25 | 1 MHz |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

The normative reference for this requirement is TS 36.101 [2] clause 6.6.2.2.2.

6.6.2.2_s.3.3	Minimum requirement (network signalled value "NS_06" or NS_07)

When "NS_06" or "NS_07" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.2.2_s.3.3-1.

Table 6.6.2.2_s.3.3-1: Additional requirements (network signalled value "NS_06" or "NS_07")

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | Measurement bandwidth |
| 0-0.1 | -13 | -13 | -15 | -18 | 30 kHz |
| 0.1-1 | -13 | -13 | -13 | -13 | 100 kHz |
| 1-2.5 | -13 | -13 | -13 | -13 | 1 MHz |
| 2.5-2.8 | -25 | -13 | -13 | -13 | 1 MHz |
| 2.8-5 |  | -13 | -13 | -13 | 1 MHz |
| 5-6 |  | -25 | -13 | -13 | 1 MHz |
| 6-10 |  |  | -25 | -13 | 1 MHz |
| 10-15 |  |  |  | -25 | 1 MHz |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

The normative reference for this requirement is TS 36.101 [2] clause 6.6.2.2.3.

6.6.2.2_s.3.4	Minimum requirement (network signalled value “NS_27”)

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When “NS_27” is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.2.2_s.3.4-1.

Table 6.6.2.2_s.3.4-1: Additional requirements (network signalled value "NS_27")

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -13 | -13 | -13 | -13 | Note 1 |
| 1-10 | -13 | -13 | -13 | -13 | 1 MHz |
| 10-15 |  | -25 | -25 | -25 | 1 MHz |
| 15-20 |  |  | -25 | -25 | 1 MHz |
| 20-25 |  |  |  | -25 | 1 MHz |
| Note 1: The measurement bandwidth is 1% of the applicable E-UTRA channel bandwidth. |  |  |  |  |  |

The normative reference for this requirement is TS 36.101 [2] clause 6.6.2.2.5.

6.6.2.2_s.3.5	Minimum requirement (network signalled value “NS_35”)

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When "NS_35" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.2.2_s.3.5-1.

Table 6.6.2.2_s.3.5-1: Additional requirements (network signalled value "NS_35")

| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth (unless otherwise stated) |
| --- | --- | --- | --- | --- | --- |
| 0-0.1 | -15 | -18 | -20 | -21 | 30 kHz |
| 0.1-6 | -13 | -13 | -13 | -13 | 100 kHz |
| 6-10 | -251 | -13 | -13 | -13 | 100 kHz |
| 10-15 |  | -251 | -13 | -13 | 100 kHz |
| 15-20 |  |  | -251 | -13 | 100 kHz |
| 20-25 |  |  |  | -25 | 1 MHz |
| Note 1: The measurement bandwidth shall be 1 MHz |  |  |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

The normative reference for this requirement is TS 36.101 [2] clause 6.6.2.2.7.

6.6.2.2_s.4	Test description

6.6.2.2_s.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Tables 6.6.2.2_s.4.1-1 through 6.6.2.2_s.4.1-9. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2 respectively. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.2_s.4.1-1: Test Configuration Table (network signalled value "NS_03")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Lowest, 5MHz, 10MHz, Highest |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |  |
| Ch BW | Mod'n | RB allocation |  | Mod'n | RB allocation |  |
|  |  | FDD | TDD |  | FDD | TDD |
| 1.4MHz | N/A for Additional Spectrum Emission Mask testing. |  |  | QPSK | 6 | 6 |
| 1.4MHz |  |  |  | QPSK | 5 | 5 |
| 1.4MHz |  |  |  | 16QAM (Note 4) | 5 | 5 |
| 3MHz |  |  |  | QPSK | 15 | 15 |
| 3MHz |  |  |  | QPSK | 4 | 4 |
| 3MHz |  |  |  | 16QAM (Note 4) | 15 | 15 |
| 3MHz |  |  |  | 16QAM (Note 4) | 4 | 4 |
| 5MHz |  |  |  | QPSK | 25 | 25 |
| 5MHz |  |  |  | QPSK | 8 | 8 |
| 5MHz |  |  |  | QPSK | 6 | 6 |
| 5MHz |  |  |  | 16QAM (Note 4) | 25 | 25 |
| 5MHz |  |  |  | 16QAM (Note 4) | 8 | 8 |
| 10MHz |  |  |  | QPSK | 50 | 50 |
| 10MHz |  |  |  | QPSK | 12 | 12 |
| 10MHz |  |  |  | QPSK | 6 | 6 |
| 10MHz |  |  |  | 16QAM (Note 4) | 50 (Note 3) | 50 (Note 3) |
| 10MHz |  |  |  | 16QAM (Note 4) | 12 | 12 |
| 15MHz |  |  |  | QPSK | 75 | 75 |
| 15MHz |  |  |  | QPSK | 16 | 16 |
| 15MHz |  |  |  | QPSK | 8 | 8 |
| 15MHz |  |  |  | 16QAM (Note 4) | 75 (Note 3) | 75 (Note 3) |
| 15MHz |  |  |  | 16QAM (Note 4) | 16 | 16 |
| 20MHz |  |  |  | QPSK | 100 | 100 |
| 20MHz |  |  |  | QPSK | 18 | 18 |
| 20MHz |  |  |  | QPSK | 10 | 10 |
| 20MHz |  |  |  | 16QAM (Note 4) | 100 (Note 3) | 100 (Note 3) |
| 20MHz |  |  |  | 16QAM (Note 4) | 18 | 18 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 3: Applies only for UE-Categories ≥2. Note 4: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |  |

Table 6.6.2.2_s.4.1-2: Test Configuration Table (network signalled value "NS_04")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range Additional test frequencies for Rel-12 to Rel-13 and bit 2 of modifiedMPRbehavior = 0: For 5 MHz bandwidth: 2499.6 MHz, NUL= 39686 For 10 MHz bandwidth: 2504.1 MHz, NUL= 39731 For 15 MHz bandwidth: 2510.9 MHz, NUL= 39799 For 20 MHz bandwidth: 2517.6 MHz, NUL= 39866 Additional test frequencies for Rel-12 and later and bit 2 of modifiedMPRbehavior = 1: For 5 MHz bandwidth: 2500.6 MHz, NUL= 39696 For 10 MHz bandwidth: 2504.1 MHz, NUL= 39731 For 15 MHz bandwidth: 2510.9 MHz, NUL= 39799 For 20 MHz bandwidth: 2517.6 MHz, NUL= 39866 |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 5MHz, 10 MHz, 15 MHz, 20MHz |  |  |
| Test Parameters for NS_04 A-MPR |  |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configura-tion ID | Ch BW | Mod'n | RB allocation TDD | Mod'n | RB allocation TDD | RBstart TDD |
| 1 (Note 6) | 5MHz | N/A for A-MPR testing |  | QPSK | 25 | 0 |
| 2 (Note 6) | 5MHz |  |  | QPSK | 8 | Note 3 |
| 3(Note 6) | 5MHz |  |  | QPSK | 6 | Note 3 |
| 4 (Note 6) | 5MHz |  |  | 16QAM (Note 5) | 25 | 0 |
| 5 (Note 6) | 5MHz |  |  | 16QAM (Note 5) | 8 | Note 3 |
| 6 (Note 6) | 10MHz |  |  | QPSK | 1 | 0 |
| 7 (Note 6) | 10MHz |  |  | QPSK | 12 | 0 |
| 8 (Note 6) | 10MHz |  |  | QPSK | 50 | 0 |
| 9 Note 6) | 10MHz |  |  | 16QAM (Note 5) | 50 (Note 4) | 0 |
| 10 (Note 6) | 10MHz |  |  | QPSK | 24 | 13 |
| 11 (Note 6) | 10MHz |  |  | 16QAM (Note 5) | 24 | 13 |
| 12 (Note 6) | 10MHz |  |  | QPSK | 36 | 13 |
| 13 (Note 6) | 10MHz |  |  | QPSK | 12 | 37 |
| 14 (Note 6) | 10MHz |  |  | QPSK | 1 | 49 |
| 15 (Note 6) | 15MHz |  |  | QPSK | 1 | 0 |
| 16 (Note 6) | 15MHz |  |  | QPSK | 16 | 0 |
| 17 (Note 6) | 15MHz |  |  | QPSK | 75 | 0 |
| 18 (Note 6) | 15MHz |  |  | 16QAM (Note 5) | 75 (Note 4) | 0 |
| 19 (Note 6) | 15MHz |  |  | QPSK | 36 | 19 |
| 20 (Note 6) | 15MHz |  |  | 16QAM (Note 5) | 36 (Note 4) | 19 |
| 21 (Note 6) | 15MHz |  |  | QPSK | 50 | 19 |
| 22 (Note 6) | 15MHz |  |  | QPSK | 18 | 56 |
| 23 (Note 6) | 15MHz |  |  | QPSK | 1 | 74 |
| 24 (Note 6) | 20MHz |  |  | QPSK | 1 | 0 |
| 25 (Note 6) | 20MHz |  |  | QPSK | 18 | 0 |
| 26 (Note 6) | 20MHz |  |  | QPSK | 100 | 0 |
| 27 (Note 6) | 20MHz |  |  | 16QAM (Note 5) | 100 (Note 4) | 0 |
| 28 (Note 6) | 20MHz |  |  | QPSK | 50 | 25 |
| 29 (Note 6) | 20MHz |  |  | 16QAM (Note 5) | 50 (Note 4) | 25 |
| 30 (Note 6) | 20MHz |  |  | QPSK | 75 | 25 |
| 31 (Note 6) | 20MHz |  |  | QPSK | 25 | 75 |
| 32 (Note 6) | 20MHz |  |  | QPSK | 1 | 99 |
| 33 (Note 8) | 5MHz |  |  | QPSK | 1 | 0 |
| 34 (Note 8) | 5MHz |  |  | QPSK | 16 | 9 |
| 35 (Note 9) | 5MHz |  |  | QPSK | 25 | 0 |
| 36 (Note 8) | 5MHz |  |  | 16QAM (Note 5) | 16 | 9 |
| 37 (Note 9) | 5MHz |  |  | 16QAM (Note 5) | 25 | 0 |
| 38 (Note 8) | 10MHz |  |  | QPSK | 1 | 0 |
| 39 (Note 8) | 10MHz |  |  | QPSK | 1 | 49 |
| 40 (Note 8) | 10MHz |  |  | QPSK | 24 | 0 |
| 41 (Note 8) | 10MHz |  |  | QPSK | 40 | 10 |
| 42 (Note 7) | 10MHz |  |  | QPSK | 50 | 0 |
| 43 (Note 7) | 10MHz |  |  | 16QAM (Note 5) | 50 | 0 |
| 44 (Note 8) | 15MHz |  |  | QPSK | 1 | 0 |
| 45 (Note 8) | 15MHz |  |  | QPSK | 1 | 74 |
| 46 (Note 8) | 15MHz |  |  | QPSK | 32 | 0 |
| 47 (Note 8) | 15MHz |  |  | QPSK | 60 | 15 |
| 48 (Note 7) | 15MHz |  |  | QPSK | 75 | 0 |
| 49 (Note 7) | 15MHz |  |  | 16QAM (Note 5) | 75 | 0 |
| 50 (Note 8) | 20MHz |  |  | QPSK | 1 | 0 |
| 51 (Note 8) | 20MHz |  |  | QPSK | 1 | 99 |
| 52 (Note 8) | 20MHz |  |  | QPSK | 36 | 0 |
| 53 (Note 8) | 20MHz |  |  | QPSK | 75 | 25 |
| 54 (Note 7) | 20MHz |  |  | QPSK | 100 | 0 |
| 55 (Note 7) | 20MHz |  |  | 16QAM (Note 5) | 100 | 0 |
| Note 1: Test Channel Bandwidths are checked separately for E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The configuration ID will be used to map the applicable Test Configuration to be corresponding Test Requirement in subclause 6.2.4 as not all combinations are necessarily required based on the applicability of the UE. Note 3: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 4: Applies only for UE-Categories ≥2. Note 5: The 16QAM configurations shall not apply to UEs which support UL 64QAM. Note 6: Applies only for Rel-11 and earlier. Note 7: Applies only for Rel-12 and later. Note 8: Applies only for Rel-12 and later and Low Range. Note 9: Applies only for Rel-12 and later and Mid, High Range and additional test frequencies. |  |  |  |  |  |  |

Table 6.6.2.2_s.4.1-3: Test Configuration Table (network signalled value "NS_06")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Lowest, 5MHz, 10MHz |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |  |
| Ch BW | Mod'n | RB allocation |  | Mod'n | RB allocation |  |
|  |  | FDD | TDD |  | FDD | TDD |
| 1.4MHz | N/A for Additional Spectrum Emission Mask testing. |  |  | QPSK | 6 | NA |
| 1.4MHz |  |  |  | QPSK | 5 |  |
| 1.4MHz |  |  |  | 16QAM (Note 3) | 5 |  |
| 3MHz |  |  |  | QPSK | 15 |  |
| 3MHz |  |  |  | QPSK | 4 |  |
| 3MHz |  |  |  | 16QAM (Note 3) | 4 |  |
| 5MHz |  |  |  | QPSK | 25 |  |
| 5MHz |  |  |  | QPSK | 8 |  |
| 5MHz |  |  |  | 16QAM (Note 3) | 8 |  |
| 10MHz |  |  |  | QPSK | 50 |  |
| 10MHz |  |  |  | QPSK | 12 |  |
| 10MHz |  |  |  | 16QAM (Note 3) | 12 |  |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 3: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |  |

Table 6.6.2.2_s.4.1-4: Test Configuration Table (network signalled value "NS_07")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  |  |  | Mid range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 10MHz |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Test Number | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD | RBstart |
| 1 | 10MHz | N/A for Additional Spectrum Emission Mask testing. |  | QPSK | 1 | 0 |
| 2 | 10MHz |  |  | QPSK | 8 | 0 |
| 3 | 10MHz |  |  | QPSK | 6 | 13 |
| 4 | 10MHz |  |  | QPSK | 20 | 13 |
| 5 | 10MHz |  |  | QPSK | 12 | 13 |
| 6 | 10MHz |  |  | 16QAM (Note 2) | 36 (Note 1) | 13 |
| 7 | 10MHz |  |  | QPSK | 16 | 19 |
| 8 | 10MHz |  |  | QPSK | 12 | 19 |
| 9 | 10MHz |  |  | 16QAM (Note 2) | 16 | 19 |
| 10 | 10MHz |  |  | QPSK | 30 | 19 |
| 11 | 10MHz |  |  | 16QAM (Note 2) | 30 (Note 1) | 19 |
| 12 | 10MHz |  |  | QPSK | 6 | 43 |
| 13 | 10MHz |  |  | QPSK | 2 | 48 |
| 14 | 10MHz |  |  | QPSK | 50 | 0 |
| 15 | 10MHz |  |  | QPSK | 12 | 0 |
| 16 | 10MHz |  |  | 16QAM (Note 2) | 50 (Note 1) | 0 |
| Note 1: Applies only for UE-Categories 2-5. Note 2: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |  |

Table 6.6.2.2_s.4.1-5: Test Configuration Table (network signalled value "NS_11")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | Normal |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range For 3 MHz Channel Bandwidth: a. 2001.5 MHz (NUL = 25515) b. 2004.5 MHz (NUL = 25545) For 5 MHz Channel Bandwidth a. 2002.5 MHz (NUL = 25525) b. 2004.5 MHz (NUL = 25545) c. 2007.5 MHz (NUL = 25575) For 10 MHz Channel Bandwidth a. 2005 MHz (NUL = 25550) b. 2005.5 MHz (NUL = 25555) c. 2015 MHz (NUL = 25650) For 15 MHz Channel Bandwidth a. 2007.5 MHz (NUL = 25575) b. 2012.5 MHz (NUL = 25625) For 20 MHz Channel Bandwidth a. 2010 MHz (NUL = 25600) |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 3MHz, 5MHz, 10MHz, 15MHz, 20MHz |  |
| Test Parameters for NS_11 A-MPR |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |
| Configuration ID | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD |
| 1 | 3MHz | N/A for A-MPR testing |  | QPSK | 6 |
| 2 | 3MHz |  |  | QPSK | 15 |
| 3 | 3MHz |  |  | 16QAM (Note 4) | 6 |
| 4 | 3MHz |  |  | 16QAM (Note 4) | 15 |
| 5 | 5MHz |  |  | QPSK | 1 |
| 6 | 5MHz |  |  | QPSK | 8 |
| 7 | 5MHz |  |  | QPSK | 25 |
| 8 | 5MHz |  |  | 16QAM (Note 4) | 8 |
| 9 | 5MHz |  |  | 16QAM (Note 4) | 25 |
| 10 | 10MHz |  |  | QPSK | 1 |
| 11 | 10MHz |  |  | QPSK | 12 |
| 12 | 10MHz |  |  | QPSK | 50 |
| 13 | 10MHz |  |  | 16QAM (Note 4) | 12 |
| 14 | 10MHz |  |  | 16QAM (Note 4) | 50 |
|  |  |  |  |  | (Note 3) |
| 15 | 15MHz |  |  | QPSK | 1 |
| 16 | 15MHz |  |  | QPSK | 8 |
| 17 | 15MHz |  |  | QPSK | 25 |
| 18 | 15MHz |  |  | QPSK | 30 |
| 19 | 15MHz |  |  | QPSK | 75 |
| 20 | 15MHz |  |  | 16QAM (Note 4) | 8 |
| 21 | 15MHz |  |  | 16QAM (Note 4) | 25 |
| 22 | 15MHz |  |  | 16QAM (Note 4) | 30 |
| 23 | 15MHz |  |  | 16QAM (Note 4) | 75 |
| 24 | 20MHz |  |  | QPSK | 1 |
| 25 | 20MHz |  |  | QPSK | 10 |
| 26 | 20MHz |  |  | QPSK | 25 |
| 27 | 20MHz |  |  | QPSK | 100 |
| 28 | 20MHz |  |  | 16QAM (Note 4) | 10 |
| 29 | 20MHz |  |  | 16QAM (Note 4) | 25 |
| 30 | 20MHz |  |  | 16QAM (Note 4) | 100 |
| Note 1: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4.5 as not all combinations are necessarily required based on the applicability of the UE. Note 2: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 3: Applies only for UE-Categories ≥2. Note 4: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |

Table 6.6.2.2_s.4.1-6: Test Configuration Table (network signalled value "NS_20")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | Normal |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range For 5 MHz Channel Bandwidth a. 2002.5 MHz (NUL = 25525) b. 2007.5 MHz (NUL = 25575) c. 2012.5 MHz (NUL = 25625) d. 2017.5 MHz (NUL = 25675) For 10 MHz Channel Bandwidth a. 2005 MHz (NUL = 25550) b. 2015 MHz (NUL = 25650) For 15 MHz Channel Bandwidth a. 2012.5 MHz (NUL = 25625) For 20 MHz Channel Bandwidth a. 2010 MHz (NUL = 25600) |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 5MHz, 10MHz, 15MHz, 20MHz |  |
| Test Parameters for NS_20 A-MPR |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |
| Configuration ID | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD |
| 1 | 5MHz | N/A for A-MPR testing |  | QPSK | 8 |
| 2 | 5MHz |  |  | QPSK | 15 |
| 3 | 5MHz |  |  | QPSK | 25 |
| 4 | 5MHz |  |  | 16QAM (Note 4) | 15 |
| 5 | 5MHz |  |  | 16QAM (Note 4) | 25 |
| 6 | 10MHz |  |  | QPSK | 8 |
| 7 | 10MHz |  |  | QPSK | 12 |
| 8 | 10MHz |  |  | QPSK | 50 |
| 9 | 10MHz |  |  | 16QAM (Note 4) | 12 |
| 10 | 10MHz |  |  | 16QAM (Note 4) | 50 |
|  |  |  |  |  | (Note 3) |
| 11 | 15MHz |  |  | QPSK | 6 |
| 12 | 15MHz |  |  | QPSK | 25 |
| 13 | 15MHz |  |  | QPSK | 36 |
| 14 | 15MHz |  |  | QPSK | 75 |
| 15 | 15MHz |  |  | 16QAM (Note 4) | 25 |
| 16 | 15MHz |  |  | 16QAM (Note 4) | 36 |
| 17 | 15MHz |  |  | 16QAM (Note 4) | 75 |
| 18 | 20MHz |  |  | QPSK | 8 |
| 19 | 20MHz |  |  | QPSK | 18 |
| 20 | 20MHz |  |  | QPSK | 25 |
| 21 | 20MHz |  |  | QPSK | 75 |
| 22 | 20MHz |  |  | QPSK | 100 |
| 23 | 20MHz |  |  | 16QAM (Note 4) | 18 |
| 24 | 20MHz |  |  | 16QAM (Note 4) | 25 |
| 25 | 20MHz |  |  | 16QAM (Note 4) | 75 |
| 26 | 20MHz |  |  | 16QAM (Note 4) | 100 |
| Note 1: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4.5 as not all combinations are necessarily required based on the applicability of the UE. Note 2: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 3: Applies only for UE-Categories ≥2. Note 4: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |

Table 6.6.2.2_s.4.1-7: Test Configuration Table (network signalled value "NS_21")

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  |  |  | NC |  |  |  |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  |  |  |  |  |  | Low range or High range |  |  |  |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  |  |  | 5 MHz, 10 MHz |  |  |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Downlink Configuration |  |  | Uplink Configuration |  |  |  |  |  |
| Configuration ID |  | Ch BW |  | Mod'n |  | RB allocation | Mod'n |  | RB allocation FDD |  | RBstart FDD |  |
| 1 (note 2) |  | 5 MHz |  |  |  |  | QPSK |  | 1 |  | 0 |  |
| 2 (note 2) |  | 5 MHz |  |  |  |  | QPSK |  | 25 |  | 0 |  |
| 3 (note 2) |  | 10 MHz |  |  |  |  | QPSK |  | 1 |  | 0 |  |
| 4 (note 3) |  | 10 MHz |  |  |  |  | QPSK |  | 1 |  | 49 |  |
| 5 (note 2) |  | 10 MHz |  |  |  |  | QPSK |  | 15 |  | 0 |  |
| 6 (note 3) |  | 10 MHz |  |  |  |  | QPSK |  | 3 |  | 47 |  |
| 7 (note 3) |  | 10 MHz |  |  |  |  | QPSK |  | 50 |  | 0 |  |
| 8 (note 1, 3) |  | 10 MHz |  |  |  |  | 16QAM (Note 4) |  | 50 |  | 0 |  |
| Note 1: Applies only for UE-Categories ≥2. Note 2: Applicable only to low range frequency testing. Note 3: Applicable only to high range frequency testing. Note 4: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.6.2.2_s.4.1-8: Test Configuration Table (network signalled value "NS_27")

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  | Normal |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | Refer to Frequency column in test parameters |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | 5MHz, 10MHz, 15MHz, 20MHz |  |  |
| Test Parameters for NS_27 A-MPR |  |  |  |  |  |  |  |
|  |  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID | Frequency | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD | RBstart FDD |
| 1 | Low/Mid | 5MHz | N/A for A-MPR testing |  | QPSK | 1 | 0 |
| 2 | High | 5MHz |  |  | QPSK | 1 | 24 |
| 3 | Low/Mid/High | 5MHz |  |  | QPSK | 25 | 0 |
| 4 | Low/Mid/High | 5MHz |  |  | 16QAM | 25 | 0 |
| 5 | Low/Mid | 10MHz |  |  | QPSK | 1 | 0 |
| 6 | High | 10MHz |  |  | QPSK | 1 | 49 |
| 7 | Low/Mid/High | 10MHz |  |  | QPSK | 50 | 0 |
| 8 (Note 2) | Low/Mid/High | 10MHz |  |  | 16QAM | 50 | 0 |
| 9 | Low/Mid | 15MHz |  |  | QPSK | 1 | 0 |
| 10 | Low | 15MHz |  |  | QPSK | 1 | 7 |
| 11 | Low/Mid | 15MHz |  |  | QPSK | 16 | 0 |
| 12 | High | 15MHz |  |  | QPSK | 1 | 74 |
| 13 | High | 15MHz |  |  | QPSK | 1 | 67 |
| 14 | High | 15MHz |  |  | QPSK | 16 | 74 |
| 15 | Low/Mid/High | 15MHz |  |  | QPSK | 75 | 0 |
| 16 (Note 2) | Low/Mid/High | 15MHz |  |  | 16QAM | 75 | 0 |
| 17 | Low/Mid | 20MHz |  |  | QPSK | 1 | 0 |
| 18 | Low | 20MHz |  |  | QPSK | 1 | 13 |
| 19 | Low | 20MHz |  |  | QPSK | 1 | 16 |
| 20 | Low/Mid | 20MHz |  |  | QPSK | 21 | 0 |
| 21 | High | 20MHz |  |  | QPSK | 1 | 99 |
| 22 | High | 20MHz |  |  | QPSK | 1 | 86 |
| 23 | High | 20MHz |  |  | QPSK | 1 | 83 |
| 24 | High | 20MHz |  |  | QPSK | 21 | 0 |
| 25 | Low/Mid/High | 20MHz |  |  | QPSK | 100 | 0 |
| 26 (Note 2) | Low/Mid/High | 20MHz |  |  | 16QAM | 100 | 0 |
| Note 1: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4.5 as not all combinations are necessarily required based on the applicability of the UE. Note 2: Applies only for UE-Categories ≥2. Note 3: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |  |  |

Table 6.6.2.2_s.4.1-9: Test Configuration Table (network signalled value "NS_35")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 5MHz, 10MHz, 15MHz, 20MHz |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |  |
| Ch BW | Mod'n | RB allocation |  | Mod'n | RB allocation |  |
|  |  | FDD | TDD |  | FDD | TDD |
| 5MHz | N/A for Additional Spectrum Emission Mask testing. |  |  | QPSK | 25 | NA |
| 5MHz |  |  |  | QPSK | 8 |  |
| 5MHz |  |  |  | 16QAM (Note 3) | 8 |  |
| 10MHz |  |  |  | QPSK | 50 |  |
| 10MHz |  |  |  | QPSK | 12 |  |
| 10MHz |  |  |  | 16QAM (Note 3) | 12 |  |
| 15MHz |  |  |  | QPSK | 75 |  |
| 15MHz |  |  |  | QPSK | 16 |  |
| 15MHz |  |  |  | 16QAM (Note 3) | 16 |  |
| 20MHz |  |  |  | QPSK | 100 |  |
| 20MHz |  |  |  | QPSK | 18 |  |
| 20MHz |  |  |  | 16QAM (Note 3) | 18 |  |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 3: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.3.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The DL Reference Measurement channels are set according to Tables 6.6.2.2_s.4.1-1 to 6.6.2.2_s.4.1-9.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.2_s.4.3.

6.6.2.2_s.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to applicable table from Table 6.6.2.2_s.4.1-1 to 6.6.2.2_s.4.1-9. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Table 6.2.4.5-1 to 6.2.4.5-28 as appropriate. The period of the measurement shall be at least the continuous duration 7 OS (0.5ms) for slot TTI and 2/3 OS for subslot TTI.

4.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to Table 6.6.2.2_s.5.1-1, 6.6.2.2_s.5.3-1 or Table 6.6.2.2_s.5.1-2, 6.6.2.2_s.5.3-2, or 6.6.2.2_s.5.2-1, 6.6.2.2_s.5.4-1, 6.6.2.2_s.5.5-1 as applicable and using a rms detector. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.6.2.2_s.4.3	Message contents

Same message contents as clause 6.6.2.2.4.3 with the following exceptions:

Table 6.6.2.2_s.4.3-1: PhysicalConfigDedicated-DEFAULT (preamble)

| Derivation Path: 36.508 Table 4.8.2.1.6-1 with condition Short-TTI |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |  |
| PhysicalConfigDedicatedSTTI-r15 SEQUENCE { |  |  |  |  |
| shortTTI-r15 SEQUENCE { |  |  |  |  |
| dl-STTI-Length-r15 |  | subslot |  | FDD_sTTI-22 OR FDD_sTTI-27 |
| ul-STTI-Length-r15 |  | subslot |  | FDD_sTTI-22 |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD_sTTI-22 | FDD cell environment and UEs supporting {subslot, subslot} combination |
| FDD_sTTI-27 | FDD cell environment and UEs supporting {subslot, slot} combination |

6.6.2.2_s.5	Test requirements

6.6.2.2_s.5.1	Test requirements (network signalled value "NS_03", “NS_11”, “NS_20” and “NS_21”)

When "NS_03" or “NS_11” or “NS_20” or “NS_21” is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.4.5-1, 6.2.4.5-2 ,6.2.4.5-11, 6.2.4.5-20 and 6.2.4.5-21 as appropriate,

and

-	the power of any UE emission shall fulfil requirements in Table 6.6.2.2_s.5.1-1 or 6.6.2.2_s.5.1-2, as applicable.

Table 6.6.2.2_s.5.1-1: Additional requirements (network signalled value "NS_03", “NS_11”, “NS_20” and “NS_21”), E-UTRA bands ≤ 3GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.5 | -11.5 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 1-2.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 |  |  |  |  | 1 MHz |
| 6-10 |  |  | -23.5 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.5 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.5 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_03 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

Table 6.6.2.2_s.5.1-2: Additional requirements (network signalled value "NS_03", “NS_11”, “NS_20” and “NS_21”), 3GHz < E-UTRA bands ≤ 4.2GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.2 | -11.2 | -13.2 | -16.2 | -18.2 | -19.2 | 30 kHz |
| 1-2.5 | -11.2 | -11.2 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 2.5-2.8 | -23.2 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.2 |  |  |  |  | 1 MHz |
| 6-10 |  |  | -23.2 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.2 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.2 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.2 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_03 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

NOTE:	(only for emission measurement) As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

6.6.2.2_s.5.2	Test requirements (network signalled value "NS_04")

When "NS_04" is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.4.5-3,

and

-	the power of any UE emission shall fulfil requirements in Table 6.6.2.2_s.5.2-1.

Table 6.6.2.2_s.5.2-1: Additional requirements (network signalled value "NS_04")

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 1-2.5 | -8.5 | --8.5 | -8.5 | -8.5 | 1 MHz |
| 2.5-2.8 |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  | 1 MHz |
| 5-6 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 6-9 | -23.5 |  |  |  | 1 MHz |
| 9-10 |  | -23.5 |  |  | 1 MHz |
| 10-13.5 |  |  |  |  | 1 MHz |
| 13.5-15 |  |  | -23.5 |  | 1 MHz |
| 15-18 |  |  |  |  | 1 MHz |
| 18-20 |  |  |  | -23.5 | 1 MHz |
| 20-25 |  |  |  |  | 1 MHz |

NOTE:	(only for emission measurement) As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

6.6.2.2_s.5.3	Test requirements (network signalled value "NS_06" or "NS_07")

When "NS_06" or "NS_07" is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.4.5-5, 6.2.4.5-6 and 6.2.4.5-7,

and

-	the power of any UE emission shall fulfil requirements in Table 6.6.2.2_s.5.3-1 or Table.6.6.2.2_s.5.3-2, as applicable.

Table 6.6.2.2_s.5.3-1: Additional requirements (network signalled value "NS_06" or "NS_07"), E-UTRA bands ≤ 3GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | Measurement bandwidth |
| 0-0.1 | -11.5 | -11.5 | -13.5 | -16.5 | 30 kHz |
| 0.1-1 | -11.5 | -11.5 | -11.5 | -11.5 | 100 kHz |
| 1-2.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 |  |  | 1 MHz |
| 6-10 |  |  | -23.5 |  | 1 MHz |
| 10-15 |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.085 MHz. The first and last measurement position with a 100 kHz filter is at ΔfOOB equals to 0.15 MHz and 0.95 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_06 and NS_07 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |

Table 6.6.2.2_s.5.3-2: Additional requirements (network signalled value "NS_06" or "NS_07"), 3GHz < E-UTRA bands ≤ 4.2GHz

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | Measurement bandwidth |
| 0-0.1 | -11.2 | -11.2 | -13.2 | -16.2 | 30 kHz |
| 0.1-1 | -11.2 | -11.2 | -11.2 | -11.2 | 100 kHz |
| 1-2.5 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 2.5-2.8 | -23.2 |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  | 1 MHz |
| 5-6 |  | -23.2 |  |  | 1 MHz |
| 6-10 |  |  | -23.2 |  | 1 MHz |
| 10-15 |  |  |  | -23.2 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.085 MHz. The first and last measurement position with a 100 kHz filter is at ΔfOOB equals to 0.15 MHz and 0.95 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: Above SEM requirement applies to bands corresponding to network signalling value NS_06 and NS_07 as defined in TS 36.101 [2] subclause 6.2.4 Table 6.2.4-1. Note 5: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |

NOTE:	(only for emission measurement) As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

6.6.2.2_s.5.4	Test requirements (network signalled value "NS_27")

When "NS_27" is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.4.5-27,

and

-	the power of any UE emission shall fulfil requirements in Table 6.6.2.2_s.5.4-1, as applicable.

Table 6.6.2.2_s.5.4-1: Additional requirements (network signalled value "NS_27")

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -11.2 | -11.2 | -11.2 | -11.2 | Note 1 |
| 1-10 | -11.2 | -11.2 | -11.2 | -11.2 | 1 MHz |
| 10-15 |  | -23.2 | -23.2 | -23.2 | 1 MHz |
| 15-20 |  |  | -23.2 | -23.2 | 1 MHz |
| 20-25 |  |  |  | -23.2 | 1 MHz |
| Note 1: The measurement bandwidth is 1% of the applicable E-UTRA channel bandwidth. |  |  |  |  |  |

6.6.2.2_s.5.5	Test requirements (network signalled value "NS_35")

When "NS_35" is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.4.5-28,

and

-	the power of any UE emission shall fulfil requirements in Table 6.6.2.2_s.5.5-1, as applicable.

Table 6.6.2.2_s.5.5-1: Additional requirements (network signalled value "NS_35")

| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth (unless otherwise stated) |
| --- | --- | --- | --- | --- | --- |
| 0-0.1 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 0.1-6 | -11.5 | -11.5 | -11.5 | -11.5 | 100 kHz |
| 6-10 | -23.51 | -11.5 | -11.5 | -11.5 | 100 kHz |
| 10-15 |  | -23.51 | -11.5 | -11.5 | 100 kHz |
| 15-20 |  |  | -23.51 | -11.5 | 100 kHz |
| 20-25 |  |  |  | -23.5 | 1 MHz |
| Note 1: The measurement bandwidth shall be 1 MHz |  |  |  |  |  |

#### 6.6.2.3	Adjacent Channel Leakage power Ratio

Editor’s Note:

- Test applicability specification needs to be updated for not applicable bands of UTRAACLR

##### 6.6.2.3.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR).

##### 6.6.2.3.2	Test applicability

This test case applies to all types of E-UTRA Power Class 3 UE release 8 and forward.

The test of UTRAACLR doesn’t apply on power class 3 UE operating in Band 7, 12, 13, 17, 20, 24, 27, 30, 33, 35, 36, 37, 38, 40, 43, 44, 45, 47, 48, 50, 51, 52, 53, 54, 68, 70, 71, 85, 87, 88, 106 and Scell operation in Band 46, 49.

##### 6.6.2.3.3	Minimum conformance requirements

ACLR requirements are specified for two scenarios for an adjacent E -UTRAACLR and UTRAACLR1/2 as shown in Figure 6.6.2.3.3-1.

Figure 6.6.2.3.3-1: Adjacent Channel Leakage Power Ratio requirements

6.6.2.3.3.1	Minimum conformance requirements for E-UTRA

E-UTRA ACLR (E-UTRAACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency at nominal channel spacing. The assigned E-UTRA channel power and adjacent E-UTRA channel power are measured with rectangular filters with measurement bandwidth specified in Table 6.6.2.3.3.1-1.

If the measured adjacent channel power is greater than –50dBm then the E-UTRAACLR shall be higher than the valued specified in Table 6.6.2.3.3.1-1.

Table 6.6.2.3.3.1-1: General requirements for E-UTRAACLR

|  | Channel bandwidth / E-UTRAACLR1 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | 30 dB | 30 dB | 30 dB | 30 dB | 30 dB | 30 dB |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |

The normative reference for this requirement is TS 36.101 [2] subclause 6.6.2.3.1.

6.6.2.3.3.2	Minimum conformance requirements for UTRA

UTRA ACLR (UTRAACLR) is the ratio of the filtered mean power centred on the assigned E-UTRA channel frequency to the filtered mean power centred on an adjacent UTRA channel frequency.

UTRA ACLR is specified for both the first UTRA adjacent channel (UTRAACLR1) and the 2nd UTRA adjacent channel (UTRAACLR2).The UTRA channel power is measured with a RRC bandwidth filter with roll-off factor = 0.22. The assigned E-UTRA channel power is measured with a rectangular filter with measurement bandwidth specified in Table 6.6.2.3.3.2-1.

If the measured UTRA channel power is greater than –50dBm then the UTRAACLR1, and UTRAACLR2 shall be higher than the valued specified in Table 6.6.2.3.3.2-1.

UTRAACLR is not applicable to the power class 3 UE operating in Band 7, 12, 13, 17, 20, 24, 27, 30, 33, 35, 36, 37, 38, 40, 43, 44, 45, 47, 48, 50, 51, 52, 53, 54, 68, 70, 71, 85, 87, 88, 106 and Scell operation in Band 46, 49.

Table 6.6.2.3.3.2-1: General requirements for UTRAACLR1/2

|  | Channel bandwidth / UTRAACLR1/2 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| UTRAACLR1 | 33 dB | 33 dB | 33 dB | 33 dB | 33 dB | 33 dB |
| Adjacent channel centre frequency offset (in MHz) | 0.7+BWUTRA/2 / -0.7-BWUTRA/2 | 1.5+BWUTRA/2 / -1.5-BWUTRA/2 | 2.5+BWUTRA/2 / -2.5-BWUTRA/2 | 5+BWUTRA/2 / -5-BWUTRA/2 | 7.5+BWUTRA/2 / -7.5-BWUTRA/2 | 10+BWUTRA/2 / -10-BWUTRA/2 |
| UTRAACLR2 | - | - | 36 dB | 36 dB | 36 dB | 36 dB |
| Adjacent channel centre frequency offset (in MHz) | - | - | 2.5+3*BWUTRA/2 / -2.5-3*BWUTRA/2 | 5+3*BWUTRA/2 / -5-3*BWUTRA/2 | 7.5+3*BWUTRA/2 / -7.5-3*BWUTRA/2 | 10+3*BWUTRA/2 / -10-3*BWUTRA/2 |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UTRA 5MHz channel Measurement bandwidth1 | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz |
| UTRA 1.6MHz channel measurement bandwidth2 | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz |
| Note 1: Applicable for E-UTRA FDD co-existence with UTRA FDD in paired spectrum. Note 2: Applicable for E-UTRA TDD co-existence with UTRA TDD in unpaired spectrum. |  |  |  |  |  |  |

The normative reference for this requirement is TS 36.101 subclause 6.6.2.3.2.

##### 6.6.2.3.4	Test description

6.6.2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.3.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in AnnexeA.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.3.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Lowest, 5MHz, 10MHz, Highest |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |  |
| Ch BW | Mod'n | RB allocation |  | Mod'n | RB allocation |  |
|  |  | FDD | TDD |  | FDD | TDD |
| 1.4MHz | N/A for ACLR testing |  |  | QPSK | 6 | 6 |
| 1.4MHz |  |  |  | QPSK | 5 | 5 |
| 1.4MHz |  |  |  | 16QAM | 6 | 6 |
| 1.4MHz |  |  |  | 16QAM | 5 | 5 |
| 3MHz |  |  |  | QPSK | 15 | 15 |
| 3MHz |  |  |  | QPSK | 4 | 4 |
| 3MHz |  |  |  | 16QAM | 15 | 15 |
| 3MHz |  |  |  | 16QAM | 4 | 4 |
| 5MHz |  |  |  | QPSK | 25 | 25 |
| 5MHz |  |  |  | QPSK | 8 | 8 |
| 5MHz |  |  |  | 16QAM | 25 | 25 |
| 5MHz |  |  |  | 16QAM | 8 | 8 |
| 10MHz |  |  |  | QPSK | 50 | 50 |
| 10MHz |  |  |  | QPSK | 12 | 12 |
| 10MHz |  |  |  | 16QAM | 50 (Note 3) | 50 (Note 3) |
| 10MHz |  |  |  | 16QAM | 12 | 12 |
| 15MHz |  |  |  | QPSK | 75 | 75 |
| 15MHz |  |  |  | QPSK | 16 | 16 |
| 15MHz |  |  |  | 16QAM | 75 (Note 3) | 75 (Note 3) |
| 15MHz |  |  |  | 16QAM | 16 | 16 |
| 20MHz |  |  |  | QPSK | 100 | 100 |
| 20MHz |  |  |  | QPSK | 18 | 18 |
| 20MHz |  |  |  | 16QAM | 100 (Note 3) | 100 (Note 3) |
| 20MHz |  |  |  | 16QAM | 18 | 18 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The RBstart of partial RB allocation shall be RB#0 and RB# (max + 1 - RB allocation) of the channel bandwidth. Note 3: Applies only for UE-Categories ≥2 |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure A.3.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.3.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.3.4.3.

6.6.2.3.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.6.2.3.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Table 6.2.3.5-1. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

4.	Measure the rectangular filtered mean power for E-UTRA using a rms detector.

5.	Measure the rectangular filtered mean power of the first E-UTRA adjacent channel on both lower and upper side of the E-UTRA channel using a rms detector, respectively.

6.	Measure the RRC filtered mean power of the first and the second UTRA adjacent channel on both lower and upper side of the E-UTRA channel using a rms detector, respectively.

7.	Calculate the ratios of the power between the values measured in step 4 over step 5 for lower and upper E-UTRAACLR, respectively.

8.	Calculated the ratios of the power between the values measured in step 4 over step 6 for lower and upper UTRAACLR1, UTRAACLR2, respectively.

6.6.2.3.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6.

##### 6.6.2.3.5	Test requirement

6.6.2.3.5.1	Test requirements E-UTRA

-	The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.3.5-1 as appropriate,

and

-	if the measured adjacent channel power is greater than –50 dBm then the measured E-UTRAACLR, derived in step 7, shall be higher than the limits in table 6.6.2.3.5.1-1.

Table 6.6.2.3.5.1-1: E-UTRA UE ACLR

|  | Channel bandwidth / E-UTRAACLR1 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UE channel | +1.4 MHz or -1.4 MHz | +3 MHz or -3 MHz | +5MHz or -5MHz | +10MHz or -10MHz | +15MHz or -15MHz | +20MHz or -20MHz |

Test requirements UTRA

If the measured UTRA channel power is greater than –50dBm then the measured UTRAACLR1, UTRAACLR2, derived in step 8, shall be higher than the limits in table 6.6.2.3.5.2-1.

Table 6.6.2.3.5.2-1: UTRA UE ACLR

|  | Channel bandwidth / UTRAACLR1/2 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| UTRAACLR1 | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB |
| Adjacent channel centre frequency offset (in MHz) | 0.7+BWUTRA/2 / -0.7-BWUTRA/2 | 1.5+BWUTRA/2 / -1.5-BWUTRA/2 | 2.5+BWUTRA/2 / -2.5-BWUTRA/2 | 5+BWUTRA/2 / -5-BWUTRA/2 | 7.5+BWUTRA/2 / -7.5-BWUTRA/2 | 10+BWUTRA/2 / -10-BWUTRA/2 |
| UTRAACLR2 | - | - | 35.2 dB | 35.2 dB | 35.2 dB | 35.2 dB |
| Adjacent channel centre frequency offset (in MHz) | - | - | 2.5+3*BWUTRA/2 / -2.5-3*BWUTRA/2 | 5+3*BWUTRA/2 / -5-3*BWUTRA/2 | 7.5+3*BWUTRA/2 / -7.5-3*BWUTRA/2 | 10+3*BWUTRA/2 / -10-3*BWUTRA/2 |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UTRA 5MHz channel Measurement bandwidth1 | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz |
| UTRA 1.6MHz channel measurement bandwidth2 | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz |
| Note 1: Applicable for E-UTRA FDD co-existence with UTRA FDD in paired spectrum. Note 2: Applicable for E-UTRA TDD co-existence with UTRA TDD in unpaired spectrum. Note 3: BWUTRA for UTRA FDD is 5MHz and for UTRA TDD is 1.6MHz. |  |  |  |  |  |  |

#### 6.6.2.3_1	Adjacent Channel Leakage power Ratio for HPUE

##### 6.6.2.3_1.1	Test purpose

To verify that HPUE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR).

##### 6.6.2.3_1.2	Test applicability

This test case applies to all types of E-UTRA Power Class 1 or Power Class 2 UE release 10 and forward.

##### 6.6.2.3_1.3	Minimum conformance requirements

ACLR requirements are specified for an adjacent E -UTRAACLR as shown in Figure 6.6.2.3_1.3-1.

Figure 6.6.2.3_1.3-1: Adjacent Channel Leakage Power Ratio requirements

6.6.2.3_1.3.1	Minimum conformance requirements for E-UTRA for HPUE

Same minimum conformance requirements as in clause 6.6.2.3.3.1 with for following exceptions:

For a power class 2 capable UE operating on Band 41, when an IE P-max as defined in TS 36.331 [5] of 23 dBm or lower is indicated in the cell or if the uplink/downlink configuration is 0 or 6, the requirements for power class 2 are not applicable, and the corresponding requirements for a power class 3 UE shall apply.

For each supported frequency band other than Band 14 and Band 41, the UE shall:

-	if the UE supports a different power class than the default UE power class for the band and the supported power class enables the higher maximum output power than that of the default power class:

-	if the band is a TDD band whose frame configuration is 0 or 6; or

-	if the IE P-Max as defined in TS 36.331 [5] is not provided; or

-	if the IE P-Max as defined in TS 36.331 [5] is provided and set to the maximum output power of the default power class or lower;

-	meet all requirements for the default power class of the operating band in which the UE is operating and set its configured transmitted power as specified in sub-clause 6.2.5;

-	else (i.e the IE P-Max as defined in TS 36.331 [5] is provided and set to the higher value than the maximum output power of the default power class):

-	meet all requirements for the supported power class and set its configured transmitted power class as specified in sub-clause 6.2.5.

-	Instead of Table 6.6.2.3.3.1-1  use Table 6.6.2.3_1.3.1-1 and 6.6.2.3_1.3.1-2

Table 6.6.2.3_1.3.1-1: Additional E-UTRAACLR requirements for Power Class 1

|  | Channel bandwidth / E-UTRAACLR1 / Measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | N/A | N/A | 37 dB | 37 dB | N/A | N/A |
| E-UTRA channel Measurement bandwidth | N/A | N/A | 4.5 MHz | 9.0 MHz | N/A | N/A |
| Adjacent channel centre frequency offset [MHz] | N/A | N/A | +5 / -5 | +10 / -10 | N/A | N/A |
| Note 1: E-UTRAACLR1 shall be applicable for >23dBm |  |  |  |  |  |  |

Table 6.6.2.3_1.3.1-2: Additional E-UTRAACLR requirements for Power Class 2

|  | Channel bandwidth / E-UTRAACLR1 / Measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | N/A | N/A | 31 dB | 31 dB | 31 dB | 31 dB |
| E-UTRA channel Measurement bandwidth | N/A | N/A | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| Adjacent channel centre frequency offset [MHz] | N/A | N/A | +5 / -5 | +10 / -10 | +15 / -15 | +20 / -20 |

The normative reference for this requirement is TS 36.101 [2] clause 6.6.2.3.

##### 6.6.2.3_1.4	Test description

6.6.2.3_1.4.1	Initial conditions

Same initial conditions as in clause 6.6.2.3.4.1.

6.6.2.3_1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.6.2.3.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Table 6.2.3_1.5-1. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

4.	Measure the rectangular filtered mean power for E-UTRA using a rms detector.

5.	Measure the rectangular filtered mean power of the first E-UTRA adjacent channel on both lower and upper side of the E-UTRA channel using a rms detector, respectively.

6.	Calculate the ratios of the power between the values measured in step 4 over step 5 for lower and upper E-UTRAACLR, respectively.

6.6.2.3_1.4.3	Message contents

Same message contents as in clause 6.6.2.3.4.3

##### 6.6.2.3_1.5	Test requirement

6.6.2.3_1.5.1	Test requirements E-UTRA

-	The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.3_1.5-1 as appropriate,

and

-	if the measured adjacent channel power is greater than –50 dBm then the measured E-UTRAACLR, derived in step 6, shall be higher than the limits in table 6.6.2.3_1.4.1-1 for Power Class1 UE and in table 6.6.2.3_1.4.1-2 for Power Class 2 UE.

Table 6.6.2.3_1.4.1-1: E-UTRA UE ACLR for Power Class 1

|  | Channel bandwidth / E-UTRAACLR1 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | N/A | N/A | 36.2 dB | 36.2 dB | N/A | N/A |
| E-UTRA channel Measurement bandwidth | N/A | N/A | 4.5 MHz | 9.0 MHz | N/A | N/A |
| UE channel | N/A | N/A | +5MHz or -5MHz | +10MHz or -10MHz | N/A | N/A |

Table 6.6.2.3_1.4.1-2: E-UTRA UE ACLR for Power Class 2

|  | Channel bandwidth / E-UTRAACLR1 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | N/A | N/A | 30.2 dB | 30.2 dB | 30.2 dB | 30.2 dB |
| E-UTRA channel Measurement bandwidth | N/A | N/A | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UE channel | N/A | N/A | +5MHz or -5MHz | +10MHz or -10MHz | +15MHz or -15MHz | +20MHz or -20MHz |

#### 6.6.2.3_2	Adjacent Channel Leakage power Ratio for Multi-Cluster PUSCH

Editor’s Note:

- Test applicability specification needs to be updated for not applicable bands of UTRAACLR

##### 6.6.2.3_2.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR) for Multi-Cluster PUSCH transmission.

##### 6.6.2.3_2.2	Test applicability

This test case applies to all types of E-UTRA UE release 10 and forward that support multi cluster PUSCH within a component carrier for the tested band but do not support UL 64QAM or UL 256QAM.

The test of UTRAACLR doesn’t apply on power class 3 UE operating in Band 7, 12, 13, 17, 20, 24, 27, 30, 33, 35, 36, 37, 38, 40, 43, 44, 45, 47, 48, 50, 51, 52, 53, 54, 68, 70, 71, 85, 87, 88, 106 and Scell operation in Band 46, 49.

##### 6.6.2.3_2.3	Minimum conformance requirements

ACLR requirements are specified for two scenarios for an adjacent E -UTRAACLR and UTRAACLR1/2 as shown in Figure 6.6.2.3_2.3-1.

Figure 6.6.2.3_2.3-1: Adjacent Channel Leakage Power Ratio requirements

6.6.2.3_2.3.1	Minimum conformance requirements for E-UTRA

E-UTRA ACLR (E-UTRAACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency at nominal channel spacing. The assigned E-UTRA channel power and adjacent E-UTRA channel power are measured with rectangular filters with measurement bandwidth specified in Table 6.6.2.3_2.3.1-1.

If the measured adjacent channel power is greater than –50dBm then the E-UTRAACLR shall be higher than the valued specified in Table 6.6.2.3_2.3.1-1.

Table 6.6.2.3_2.3.1-1: General requirements for E-UTRAACLR

|  | Channel bandwidth / E-UTRAACLR1 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | 30 dB | 30 dB | 30 dB | 30 dB | 30 dB | 30 dB |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |

The normative reference for this requirement is TS 36.101 [2] subclause 6.6.2.3.1.

6.6.2.3_2.3.2	Minimum conformance requirements for UTRA

UTRA ACLR (UTRAACLR) is the ratio of the filtered mean power centred on the assigned E-UTRA channel frequency to the filtered mean power centred on an adjacent UTRA channel frequency.

UTRA ACLR is specified for both the first UTRA adjacent channel (UTRAACLR1) and the 2nd UTRA adjacent channel (UTRAACLR2).The UTRA channel power is measured with a RRC bandwidth filter with roll-off factor = 0.22. The assigned E-UTRA channel power is measured with a rectangular filter with measurement bandwidth specified in Table 6.6.2.3_2.3.2-1.

If the measured UTRA channel power is greater than –50dBm then the UTRAACLR1, and UTRAACLR2 shall be higher than the valued specified in Table 6.6.2.3_2.3.2-1.

UTRAACLR is not applicable to the power class 3 UE operating in Band 7, 12, 13, 17, 20, 24, 27, 30, 33, 35, 36, 37, 38, 40, 43, 44, 45, 47, 48, 50, 51, 52, 53, 54, 68, 70, 71, 85, 87, 88, 106 and Scell operation in Band 46, 49.

Table 6.6.2.3_2.3.2-1: General requirements for UTRAACLR1/2

|  | Channel bandwidth / UTRAACLR1/2 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| UTRAACLR1 | 33 dB | 33 dB | 33 dB | 33 dB | 33 dB | 33 dB |
| Adjacent channel centre frequency offset (in MHz) | 0.7+BWUTRA/2 / -0.7-BWUTRA/2 | 1.5+BWUTRA/2 / -1.5-BWUTRA/2 | 2.5+BWUTRA/2 / -2.5-BWUTRA/2 | 5+BWUTRA/2 / -5-BWUTRA/2 | 7.5+BWUTRA/2 / -7.5-BWUTRA/2 | 10+BWUTRA/2 / -10-BWUTRA/2 |
| UTRAACLR2 | - | - | 36 dB | 36 dB | 36 dB | 36 dB |
| Adjacent channel centre frequency offset (in MHz) | - | - | 2.5+3*BWUTRA/2 / -2.5-3*BWUTRA/2 | 5+3*BWUTRA/2 / -5-3*BWUTRA/2 | 7.5+3*BWUTRA/2 / -7.5-3*BWUTRA/2 | 10+3*BWUTRA/2 / -10-3*BWUTRA/2 |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UTRA 5MHz channel Measurement bandwidth1 | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz |
| UTRA 1.6MHz channel measurement bandwidth2 | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz |
| Note 1: Applicable for E-UTRA FDD co-existence with UTRA FDD in paired spectrum. Note 2: Applicable for E-UTRA TDD co-existence with UTRA TDD in unpaired spectrum. |  |  |  |  |  |  |

The normative reference for this requirement is TS 36.101 subclause 6.6.2.3.2.

##### 6.6.2.3_2.4	Test description

6.6.2.3_2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.3_2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in AnnexeA.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.3_2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 |  |  |  | Low range, Mid range, High range |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  |  | Highest |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |  |
| Configuration ID | Downlink Configuration |  | Uplink Configuration |  |  |  |  |
|  | Ch BW | N/A for SEM testing | Mod'n |  | Cluster1 RB allocations (LCRB @ RBstart) |  | Cluster2 RB allocations (LCRB @ RBstart) |
| 1 | 5MHz |  | 16QAM |  | 2@0 | 1@24 |  |
| 2 | 5MHz |  | 16QAM |  | 18@0 | 2@22 |  |
| 3 | 5MHz |  | 16QAM |  | 2@0 | 18@6 |  |
| 4 | 10MHz |  | 16QAM |  | 3@0 | 2@48 |  |
| 5 | 10MHz |  | 16QAM |  | 42@0 | 3@45 |  |
| 6 | 10MHz |  | 16QAM |  | 3@0 | 42@6 |  |
| 7 | 15MHz |  | 16QAM |  | 8@0 | 7@68 |  |
| 8 | 15MHz |  | 16QAM |  | 60@0 | 4@68 |  |
| 9 | 15MHz |  | 16QAM |  | 4@0 | 60@12 |  |
| 10 | 20MHz |  | 16QAM |  | 4@0 | 4@96 |  |
| 11 | 20MHz |  | 16QAM |  | 92@0 | 4@96 |  |
| 12 | 20MHz |  | 16QAM |  | 4@0 | 92@8 |  |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure A.3.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.3_2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.3_2.4.3.

6.6.2.3_2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.6.2.3_2.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Table 6.2.3_2.5-1. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

4.	Measure the rectangular filtered mean power for E-UTRA using a rms detector.

5.	Measure the rectangular filtered mean power of the first E-UTRA adjacent channel on both lower and upper side of the E-UTRA channel using a rms detector, respectively.

6.	Measure the RRC filtered mean power of the first and the second UTRA adjacent channel on both lower and upper side of the E-UTRA channel using a rms detector, respectively.

7.	Calculate the ratios of the power between the values measured in step 4 over step 5 for lower and upper E-UTRAACLR, respectively.

8.	Calculated the ratios of the power between the values measured in step 4 over step 6 for lower and upper UTRAACLR1, UTRAACLR2, respectively.

6.6.2.3_2.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6.

##### 6.6.2.3_2.5	Test requirement

6.6.2.3_2.5.1	Test requirements E-UTRA

-	The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.3_2.5-1 as appropriate,

and

-	if the measured adjacent channel power is greater than –50 dBm then the measured E-UTRAACLR, derived in step 7, shall be higher than the limits in table 6.6.2.3_2.5.1-1.

Table 6.6.2.3_2.5.1-1: E-UTRA UE ACLR

|  | Channel bandwidth / E-UTRAACLR1 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UE channel | +1.4 MHz or -1.4 MHz | +3 MHz or -3 MHz | +5MHz or -5MHz | +10MHz or -10MHz | +15MHz or -15MHz | +20MHz or -20MHz |

6.6.2.3_2.5.2	Test requirements UTRA

If the measured UTRA channel power is greater than –50dBm then the measured UTRAACLR1, UTRAACLR2, derived in step 8, shall be higher than the limits in table 6.6.2.3_2.5.2-1.

Table 6.6.2.3_2.5.2-1: UTRA UE ACLR

|  | Channel bandwidth / UTRAACLR1/2 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| UTRAACLR1 | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB |
| Adjacent channel centre frequency offset (in MHz) | 0.7+BWUTRA/2 / -0.7-BWUTRA/2 | 1.5+BWUTRA/2 / -1.5-BWUTRA/2 | 2.5+BWUTRA/2 / -2.5-BWUTRA/2 | 5+BWUTRA/2 / -5-BWUTRA/2 | 7.5+BWUTRA/2 / -7.5-BWUTRA/2 | 10+BWUTRA/2 / -10-BWUTRA/2 |
| UTRAACLR2 | - | - | 35.2 dB | 35.2 dB | 35.2 dB | 35.2 dB |
| Adjacent channel centre frequency offset (in MHz) | - | - | 2.5+3*BWUTRA/2 / -2.5-3*BWUTRA/2 | 5+3*BWUTRA/2 / -5-3*BWUTRA/2 | 7.5+3*BWUTRA/2 / -7.5-3*BWUTRA/2 | 10+3*BWUTRA/2 / -10-3*BWUTRA/2 |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UTRA 5MHz channel Measurement bandwidth1 | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz |
| UTRA 1.6MHz channel measurement bandwidth2 | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz |
| Note 1: Applicable for E-UTRA FDD co-existence with UTRA FDD in paired spectrum. Note 2: Applicable for E-UTRA TDD co-existence with UTRA TDD in unpaired spectrum. Note 3: BWUTRA for UTRA FDD is 5MHz and for UTRA TDD is 1.6MHz. |  |  |  |  |  |  |

#### 6.6.2.3_3	Adjacent Channel Leakage power Ratio for UL 64QAM

##### 6.6.2.3_3.1	Test purpose

Same test purpose as in clause 6.6.2.3.1.

##### 6.6.2.3_3.2	Test applicability

This test case applies to all types of E-UTRA UE from release 13 and forward that support UL 64QAM.

##### 6.6.2.3_3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.6.2.3.3.

##### 6.6.2.3_3.4	Test description

6.6.2.3_3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.3_3.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in AnnexeA.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.3_3.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Lowest, 5MHz, 10MHz, Highest |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |  |
| Ch BW | Mod'n | RB allocation |  | Mod'n | RB allocation |  |
|  |  | FDD | TDD |  | FDD | TDD |
| 1.4MHz |  |  |  | 64QAM | 6 | 6 |
| 1.4MHz |  |  |  | 64QAM | 5 | 5 |
| 3MHz |  |  |  | 64QAM | 15 | 15 |
| 3MHz |  |  |  | 64QAM | 4 | 4 |
| 5MHz |  |  |  | 64QAM | 25 | 25 |
| 5MHz |  |  |  | 64QAM | 8 | 8 |
| 10MHz |  |  |  | 64QAM | 50 | 50 |
| 10MHz |  |  |  | 64QAM | 12 | 12 |
| 15MHz |  |  |  | 64QAM | 75 | 75 |
| 15MHz |  |  |  | 64QAM | 16 | 16 |
| 20MHz |  |  |  | 64QAM | 100 | 100 |
| 20MHz |  |  |  | 64QAM | 18 | 18 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The RBstart of partial RB allocation shall be RB#0 and RB# (max + 1 - RB allocation) of the channel bandwidth. |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure A.3.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.3_3.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.3_3.4.3.

6.6.2.3_3.4.2	Test procedure

Same test procedure as in clause 6.6.2.3.4.2 with the following exceptions

- In step 3, instead of Table 6.2.3.5-1 use Table 6.2.3_3.5-1.

6.6.2.3_3.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6.

##### 6.6.2.3_3.5	Test requirement

6.6.2.3_3.5.1	Test requirements E-UTRA

-	The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.3_1.5-1 as appropriate, and

-	if the measured adjacent channel power is greater than –50 dBm then the measured E-UTRAACLR, derived in step 7, shall be higher than the limits in table 6.6.2.3_3.5.1-1.

Table 6.6.2.3_3.5.1-1: E-UTRA UE ACLR

|  | Channel bandwidth / E-UTRAACLR1 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UE channel | +1.4 MHz or -1.4 MHz | +3 MHz or -3 MHz | +5MHz or -5MHz | +10MHz or -10MHz | +15MHz or -15MHz | +20MHz or -20MHz |

6.6.2.3_3.5.2	Test requirements UTRA

If the measured UTRA channel power is greater than –50dBm then the measured UTRAACLR1, UTRAACLR2, derived in step 8, shall be higher than the limits in table 6.6.2.3_3.5.2-1.

Table 6.6.2.3_3.5.2-1: UTRA UE ACLR

|  | Channel bandwidth / UTRAACLR1/2 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| UTRAACLR1 | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB |
| Adjacent channel centre frequency offset (in MHz) | 0.7+BWUTRA/2 / -0.7-BWUTRA/2 | 1.5+BWUTRA/2 / -1.5-BWUTRA/2 | 2.5+BWUTRA/2 / -2.5-BWUTRA/2 | 5+BWUTRA/2 / -5-BWUTRA/2 | 7.5+BWUTRA/2 / -7.5-BWUTRA/2 | 10+BWUTRA/2 / -10-BWUTRA/2 |
| UTRAACLR2 | - | - | 35.2 dB | 35.2 dB | 35.2 dB | 35.2 dB |
| Adjacent channel centre frequency offset (in MHz) | - | - | 2.5+3*BWUTRA/2 / -2.5-3*BWUTRA/2 | 5+3*BWUTRA/2 / -5-3*BWUTRA/2 | 7.5+3*BWUTRA/2 / -7.5-3*BWUTRA/2 | 10+3*BWUTRA/2 / -10-3*BWUTRA/2 |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UTRA 5MHz channel Measurement bandwidth1 | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz |
| UTRA 1.6MHz channel measurement bandwidth2 | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz |
| Note 1: Applicable for E-UTRA FDD co-existence with UTRA FDD in paired spectrum. Note 2: Applicable for E-UTRA TDD co-existence with UTRA TDD in unpaired spectrum. Note 3: BWUTRA for UTRA FDD is 5MHz and for UTRA TDD is 1.6MHz. |  |  |  |  |  |  |

#### 6.6.2.3_4	Adjacent Channel Leakage power Ratio for Multi-Cluster PUSCH with UL 64QAM

Editor's note: For a transition period until RAN5#83, this test case in version 15.3.1 of 36.521-1 shall be used. This is to ensure no test coverage is lost before the UL 256QAM test case becomes available.

##### 6.6.2.3_4.1	Test purpose

Same test purpose as in clause 6.6.2.3_2.

##### 6.6.2.3_4.2	Test applicability

This test case applies to all types of E-UTRA UE from release 13 and forward that support UL 64QAM and multi cluster PUSCH within a component carrier for the tested band but do not support UL 256QAM.

##### 6.6.2.3_4.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.6.2.3_2.3.

##### 6.6.2.3_4.4	Test description

6.6.2.3_4.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.3_4.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in AnnexeA.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.3_4.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 |  |  |  | Low range, Mid range, High range |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  |  | Highest |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |  |
| Configuration ID | Downlink Configuration |  | Uplink Configuration |  |  |  |  |
|  | Ch BW | N/A for SEM testing | Mod'n |  | Cluster1 RB allocations (LCRB @ RBstart) |  | Cluster2 RB allocations (LCRB @ RBstart) |
| 1 | 5MHz |  | 64QAM (Note 2) |  | 2@0 | 1@24 |  |
| 2 | 5MHz |  | 64QAM |  | 18@0 | 2@22 |  |
| 3 | 5MHz |  | 64QAM |  | 2@0 | 18@6 |  |
| 4 | 10MHz |  | 64QAM (Note 2) |  | 3@0 | 2@48 |  |
| 5 | 10MHz |  | 64QAM |  | 42@0 | 3@45 |  |
| 6 | 10MHz |  | 64QAM |  | 3@0 | 42@6 |  |
| 7 | 15MHz |  | 64QAM (Note 2) |  | 8@0 | 7@68 |  |
| 8 | 15MHz |  | 64QAM |  | 60@0 | 4@68 |  |
| 9 | 15MHz |  | 64QAM |  | 4@0 | 60@12 |  |
| 10 | 20MHz |  | 64QAM (Note 2) |  | 4@0 | 4@96 |  |
| 11 | 20MHz |  | 64QAM |  | 92@0 | 4@96 |  |
| 12 | 20MHz |  | 64QAM |  | 4@0 | 92@8 |  |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The 64QAM configurations shall not apply to UEs for which TC 6.6.2.3_6 is executed. |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure A.3.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.3_4.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.3_4.4.3.

6.6.2.3_4.4.2	Test procedure

Same test procedure as in clause 6.6.2.3_2.4.2 with following exception:

-	In step 3, instead of Table 6.2.3_2.5-1  use Table 6.2.3_4.5-1

6.6.2.3_4.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6.

##### 6.6.2.3_4.5	Test requirement

Same test requirements as in clause 6.6.2.3_2.5 with following exception:

-	Instead of Table 6.2.3_2.5-1  use Table 6.2.3_4.5-1

#### 6.6.2.3_5	Adjacent Channel Leakage power Ratio for UL 256QAM

##### 6.6.2.3_5.1	Test purpose

Same test purpose as in clause 6.6.2.3.1.

##### 6.6.2.3_5.2	Test applicability

This test case applies to all types of E-UTRA UE from release 14 and forward that support UL 256QAM.

##### 6.6.2.3_5.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.6.2.3.3.

##### 6.6.2.3_5.4	Test description

6.6.2.3_5.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.3_5.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in AnnexeA.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.3_5.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Lowest, 5MHz, 10MHz, Highest |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |  |
| Ch BW | Mod'n | RB allocation |  | Mod'n | RB allocation |  |
|  |  | FDD | TDD |  | FDD | TDD |
| 1.4MHz |  |  |  | 256QAM | 6 | 6 |
| 3MHz |  |  |  | 256QAM | 15 | 15 |
| 5MHz |  |  |  | 256QAM | 25 | 25 |
| 10MHz |  |  |  | 256QAM | 50 | 50 |
| 15MHz |  |  |  | 256QAM | 75 | 75 |
| 20MHz |  |  |  | 256QAM | 100 | 100 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in Table 5.4.2.1-1. |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure A.3.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.3_5.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.3_5.4.3.

6.6.2.3_5.4.2	Test procedure

Same test procedure as in clause 6.6.2.3.4.2 with the following exceptions

- In step 3, instead of Table 6.2.3.5-1 use Table 6.2.3_5.5-1

6.6.2.3_5.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6.

##### 6.6.2.3_5.5	Test requirement

6.6.2.3_5.5.1	Test requirements E-UTRA

-	The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.3_5.5-1 as appropriate, and

-	if the measured adjacent channel power is greater than –50 dBm then the measured E-UTRAACLR, derived in step 7, shall be higher than the limits in table 6.6.2.3_5.5.1-1.

Table 6.6.2.3_5.5.1-1: E-UTRA UE ACLR

|  | Channel bandwidth / E-UTRAACLR1 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UE channel | +1.4 MHz or -1.4 MHz | +3 MHz or -3 MHz | +5MHz or -5MHz | +10MHz or -10MHz | +15MHz or -15MHz | +20MHz or -20MHz |

6.6.2.3_5.5.2	Test requirements UTRA

If the measured UTRA channel power is greater than –50dBm then the measured UTRAACLR1, UTRAACLR2, derived in step 8, shall be higher than the limits in table 6.6.2.3_5.5.2-1.

Table 6.6.2.3_5.5.2-1: UTRA UE ACLR

|  | Channel bandwidth / UTRAACLR1/2 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| UTRAACLR1 | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB |
| Adjacent channel centre frequency offset (in MHz) | 0.7+BWUTRA/2 / -0.7-BWUTRA/2 | 1.5+BWUTRA/2 / -1.5-BWUTRA/2 | 2.5+BWUTRA/2 / -2.5-BWUTRA/2 | 5+BWUTRA/2 / -5-BWUTRA/2 | 7.5+BWUTRA/2 / -7.5-BWUTRA/2 | 10+BWUTRA/2 / -10-BWUTRA/2 |
| UTRAACLR2 | - | - | 35.2 dB | 35.2 dB | 35.2 dB | 35.2 dB |
| Adjacent channel centre frequency offset (in MHz) | - | - | 2.5+3*BWUTRA/2 / -2.5-3*BWUTRA/2 | 5+3*BWUTRA/2 / -5-3*BWUTRA/2 | 7.5+3*BWUTRA/2 / -7.5-3*BWUTRA/2 | 10+3*BWUTRA/2 / -10-3*BWUTRA/2 |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UTRA 5MHz channel Measurement bandwidth1 | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz |
| UTRA 1.6MHz channel measurement bandwidth2 | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz |
| Note 1: Applicable for E-UTRA FDD co-existence with UTRA FDD in paired spectrum. Note 2: Applicable for E-UTRA TDD co-existence with UTRA TDD in unpaired spectrum. Note 3: BWUTRA for UTRA FDD is 5MHz and for UTRA TDD is 1.6MHz. |  |  |  |  |  |  |

#### 6.6.2.3_6	Adjacent Channel Leakage power Ratio for Multi-Cluster PUSCH with UL 256QAM

##### 6.6.2.3_6.1	Test purpose

Same test purpose as in clause 6.6.2.3_2.

##### 6.6.2.3_6.2	Test applicability

This test case applies to all types of E-UTRA UE from release 14 and forward that support UL 256QAM and multi cluster PUSCH within a component carrier for the tested band.

##### 6.6.2.3_6.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.6.2.3_2.3.

##### 6.6.2.3_6.4	Test description

6.6.2.3_6.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.3_6.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in AnnexeA.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.3_6.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 |  |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  |  | Highest |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
| Configuration ID | Downlink Configuration |  | Uplink Configuration |  |  |  |
|  | Ch BW | RB allocations | Mod’n |  | Cluster1 RB allocations (LCRB @ RBstart) | Cluster2 RB allocations (LCRB @ RBstart) |
| 1 | 5MHz | N/A for | 256QAM |  | 2@0 | 1@24 |
| 2 | 5MHz | ACLR | 256QAM |  | 18@0 | 2@22 |
| 3 | 5MHz | Test case | 256QAM |  | 2@0 | 18@6 |
| 4 | 10MHz |  | 256QAM |  | 3@0 | 2@48 |
| 5 | 10MHz |  | 256QAM |  | 42@0 | 3@45 |
| 6 | 10MHz |  | 256QAM |  | 3@0 | 42@6 |
| 7 | 15MHz |  | 256QAM |  | 8@0 | 7@68 |
| 8 | 15MHz |  | 256QAM |  | 60@0 | 4@68 |
| 9 | 15MHz |  | 256QAM |  | 4@0 | 60@12 |
| 10 | 20MHz |  | 256QAM |  | 4@0 | 4@96 |
| 11 | 20MHz |  | 256QAM |  | 92@0 | 4@96 |
| 12 | 20MHz |  | 256QAM |  | 4@0 | 92@8 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure A.3.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.3_6.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.3_6.4.3.

6.6.2.3_6.4.2	Test procedure

Same test procedure as in clause 6.6.2.3_2.4.2 with following exception:

-	In step 1, instead of Table 6.6.2.3_2.4.1-1  use Table 6.6.2.3_6.4.1-1

-	In step 3, instead of Table 6.2.3_2.5-1  use Table 6.2.3_6.5-1

6.6.2.3_6.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6.

##### 6.6.2.3_6.5	Test requirement

Same test requirements as in clause 6.6.2.3_2.5 with following exception:

-	Instead of Table 6.2.3_2.5-1  use Table 6.2.3_6.5-1

#### 6.6.2.3A	Adjacent Channel Leakage power Ratio for CA

##### 6.6.2.3A.0	Minimum conformance requirements

ACLR for CA requirements are specified for two scenarios for an adjacent E -UTRAACLR and UTRAACLR1/2 as shown in Figure 6.6.2.3A.0-1.

Figure 6.6.2.3A.0-1: Adjacent Channel Leakage Power Ratio for CA requirements

###### 6.6.2.3A.0.1	Minimum requirement UTRA for CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink active in two E-UTRA bands, the UTRA Adjacent Channel Leakage power Ratio (UTRAACLR) is the ratio of the filtered mean power centred on the assigned channel bandwidth on the component carrier to the filtered mean power centred on an adjacent channel frequency. The UTRA Adjacent Channel Leakage power Ratio is defined per carrier and the requirement is specified in subclause 6.6.2.3.3.

For intra-band contiguous carrier aggregation the UTRA Adjacent Channel Leakage power Ratio (UTRAACLR) is the ratio of the filtered mean power centred on the aggregated channel bandwidth to the filtered mean power centred on an adjacent(s) UTRA channel frequency.

For intra-band non-contiguous carrier aggregation when all sub-blocks consist of one component carrier the UTRA Adjacent Channel Leakage power Ratio (UTRAACLR) is the ratio of the sum of the filtered mean powers centred on the assigned sub-block frequencies to the filtered mean power centred on an adjacent(s) UTRA channel frequency. UTRAACLR1/2 requirements are applicable for all sub-blocks and are specified in Table 6.6.2.3A.0.1-2. UTRAACLR1 is required to be met in the sub-block gap when the gap bandwidth Wgap is 5MHz≤Wgap <15MHz. Both UTRAACLR1 and UTRAACLR2 are required to be met in the sub-block gap when the gap bandwidth Wgap is 15MHz≤Wgap.

For combinations of intra-band and inter-band carrier aggregation with three uplink component carriers (up to two contiguously aggregated carriers per band), the UTRA Adjacent Channel Leakage power Ratio (UTRAACLR) is defined as follows. For the E-UTRA band supporting one component carrier, the UTRA Adjacent Channel Leakage power Ratio (UTRAACLR) is the ratio of the filtered mean power centred on the assigned channel bandwidth of the component carrier to the filtered mean power centred on an adjacent(s) UTRA channel frequency and the requirements specified in subclause 6.6.2.3.3 apply. For the E-UTRA band supporting two contiguous component carriers the UTRA Adjacent Channel Leakage power Ratio (UTRAACLR) is the ratio of the filtered mean power centred on the aggregated channel bandwidth to the filtered mean power centred on an adjacent(s) UTRA channel frequency and the requirements specified in subclause 6.6.2.3A.0.1 apply.

UTRA Adjacent Channel Leakage power Ratio is specified for both the first UTRA adjacent channel (UTRAACLR1) and the 2nd UTRA adjacent channel (UTRAACLR2). The UTRA channel power is measured with a RRC bandwidth filter with roll-off factor =0.22. The assigned aggregated channel bandwidth power is measured with a rectangular filter with measurement bandwidth specified in Table 6.6.2.3A.0.1-1 for intraband contiguous carrier aggregation or 6.6.2.3A.0.1-2 for intraband non-contiguous carrier aggregation. If the measured UTRA channel power is greater than –50dBm then the UTRAACLR shall be higher than the value specified in Table 6.6.2.3A.0.1-1 for intraband contiguous carrier aggregation or 6.6.2.3A.0.1-2 for intraband non-contiguous carrier aggregation.

For carrier aggregation with one or two uplink component carriers, the UTRAACLR requirements for the PC3 UE are not applicable to the uplink component carrier(s) assigned to one of the E-UTRA band in Band 7, 12, 13, 17, 20, 24, 27, 30, 33, 35, 36, 37, 38, 40, 43, 44, 45, 46, 47, 48, 49, 50, 51, 52, 68, 70, 71 or 85.

Table 6.6.2.3A.0.1-1: Requirements for UTRAACLR1/2

|  | CA bandwidth class / UTRAACLR1/2 / measurement bandwidth |
| --- | --- |
|  | CA bandwidth class B and C |
| UTRAACLR1 | 33 dB |
| Adjacent channel centre frequency offset (in MHz) | + BWChannel_CA /2 + BWUTRA/2 / - BWChannel_CA / 2 - BWUTRA/2 |
| UTRAACLR2 | 36 dB |
| Adjacent channel centre frequency offset (in MHz) | + BWChannel_CA /2 + 3*BWUTRA/2 / - BWChannel_CA /2 – 3*BWUTRA/2 |
| CA E-UTRA channel Measurement bandwidth | BWChannel_CA - 2* BWGB |
| UTRA 5MHz channel Measurement bandwidth (Note 1) | 3.84 MHz |
| UTRA 1.6MHz channel measurement bandwidth (Note 2) | 1.28 MHz |
| NOTE 1: Applicable for E-UTRA FDD co-existence with UTRA FDD in paired spectrum. NOTE 2: Applicable for E-UTRA TDD co-existence with UTRA TDD in unpaired spectrum. |  |

Table 6.6.2.3A.0.1-2: Requirements for intraband non-contiguous CA UTRAACLR1/2

|  | UTRAACLR1/2 / measurement bandwidth |
| --- | --- |
| UTRAACLR1 | 33 dB |
| Adjacent channel centre frequency offset (in MHz) | + Fedge,block,high + BWUTRA/2 / - Fedge,block,low - BWUTRA/2 |
| UTRAACLR2 | 36 dB |
| Adjacent channel centre frequency offset (in MHz) | + Fedge,block,high + 3*BWUTRA/2 / - Fedge,block,low – 3*BWUTRA/2 |
| Sub-block measurement bandwidth | BWChannel,block - 2* BWGB |
| UTRA 5 MHz channel Measurement bandwidth (Note 1) | 3.84 MHz |
| UTRA 1.6 MHz channel measurement bandwidth (Note 2) | 1.28 MHz |
| NOTE 1: Applicable for E-UTRA FDD co-existence with UTRA FDD in paired spectrum. NOTE 2: Applicable for E-UTRA TDD co-existence with UTRA TDD in unpaired spectrum. |  |

The normative reference for this requirement is TS 36.101 [2] subclause 6.6.2.3.2A.

###### 6.6.2.3A.0.2	Minimum requirements for CA E-UTRA

For intra-band contiguous carrier aggregation the carrier aggregation E-UTRA Adjacent Channel Leakage power Ratio (CA E-UTRAACLR) is the ratio of the filtered mean power centred on the aggregated channel bandwidth to the filtered mean power centred on an adjacent aggregated channel bandwidth at nominal channel spacing. The assigned aggregated channel bandwidth power and adjacent aggregated channel bandwidth power are measured with rectangular filters with measurement bandwidths specified in Table 6.6.2.3A.0.2-1. If the measured adjacent channel power is greater than –50dBm then the E-UTRAACLR shall be higher than the value specified in Table 6.6.2.3A.0.2-1 and Table 6.6.2.3A.0.2-2.

Table 6.6.2.3A.0.2-1: General requirements for CA E-UTRAACLR

|  | CA bandwidth class / CA E-UTRAACLR / Measurement bandwidth |
| --- | --- |
|  | CA bandwidth class B, C and D |
| CA E-UTRAACLR | 30 dB |
| CA E-UTRA channel Measurement bandwidth | BWChannel_CA - 2* BWGB |
| Adjacent channel centre frequency offset (in MHz) | + BWChannel_CA / - BWChannel_CA |

Table 6.6.2.3A.0.2-2: Additional requirements for CA E-UTRAACLR for UL CA_41C Power Class 2

|  | CA bandwidth class / CA E-UTRAACLR / Measurement bandwidth |
| --- | --- |
|  | CA bandwidth class C |
| CA E-UTRAACLR | 31 dB |
| CA E-UTRA channel Measurement bandwidth | BWChannel_CA - 2* BWGB |
| Adjacent channel centre frequency offset (in MHz) | + BWChannel_CA / - BWChannel_CA |

For inter-band carrier aggregation with one component carrier per operating band and the uplink active in two E-UTRA bands, E-UTRA Adjacent Channel Leakage power Ratio (E-UTRAACLR) is the ratio of the filtered mean power centred on the assigned channel bandwidth on a component carrier to the filtered mean power centred on an adjacent channel frequency. The E-UTRA Adjacent Channel Leakage power Ratio is defined per carrier and the requirement is specified in subclause 6.6.2.3.1.

For intra-band non-contiguous carrier aggregation when all sub-blocks consist of one component carrier the E-UTRA Adjacent Channel Leakage power Ratio (E-UTRAACLR) is the ratio of the sum of the filtered mean powers centred on the assigned sub-block frequencies to the filtered mean power centred on an adjacent channel frequency at nominal channel spacing. In case the sub-block gap bandwidth Wgap is smaller than of the sub-block bandwidth then for that sub-block no E-UTRAACLR requirement is set for the gap. In case the sub-block gab bandwidth Wgap is smaller than either of the sub-block bandwidths then no E- UTRAACLR requirement is set for the gap. The assigned E-UTRA sub-block power and adjacent E-UTRA channel power are measured with rectangular filters with measurement bandwidths specified in Table 6.6.2.3A.0.2-2. If the measured adjacent channel power is greater than –50dBm then the E-UTRAACLR shall be higher than the value specified in Table 6.6.2.3A.0.2-2.

Table 6.6.2.3A.0.2-2: General requirements for non-contiguous intraband CA E-UTRAACLR

|  | CC and adjacent channel bandwidth / E-UTRAACLR / Measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | 30 dB | 30 dB | 30 dB | 30 dB | 30 dB | 30 dB |
| CC and adjacent channel measurement bandwidth [MHz] | 1.08 | 2.7 | 4.5 | 9 | 13.5 | 18 |
| Adjacent channel centre frequency offset [MHz] | + 1.4 / - 1.4 | + 3 / - 3 | + 5 / - 5 | + 10 / - 10 | + 15 / - 15 | + 20 / - 20 |

For combinations of intra-band and inter-band carrier aggregation with three uplink component carriers (up to two contiguously aggregated carriers per band), the E-UTRA Adjacent Channel Leakage power Ratio (E-UTRAACLR) is defined as follows. For the E-UTRA band supporting one component carrier, the E-UTRA Adjacent Channel Leakage power Ratio (UTRAACLR) is the ratio of the filtered mean power centred on the assigned channel bandwidth of the component carrier to the filtered mean power centred on an adjacent channel frequency and the requirements in subclause 6.6.2.3.3 apply. For the E-UTRA band supporting two contiguous component carriers the E-UTRA Adjacent Channel Leakage power Ratio (E-UTRAACLR) is the ratio of the filtered mean power centred on the aggregated channel bandwidth to the filtered mean power centred on an adjacent(s) aggregated channel bandwidth at nominal channel spacing and the requirements of CA E-UTRAACLR specified in subclause 6.6.2.3A.0.2 apply.

The normative reference for this requirement is TS 36.101 [2] subclause 6.6.2.3.3A.

6.6.2.3A.0.3	Additional minimum requirement for E-UTRA (network signalled value “NS_29”)

When "NS_29" is indicated in the cell, the UE emission shall meet the additional requirements specified in Table 6.6.2.3A.0.3-1 for E-UTRA channels assigned within the frequency ranges 5150-5350 MHz and 5470-5725 MHz. The assigned E-UTRA channel power and alternative adjacent E-UTRA channel power are measured with rectangular filters with measurement bandwidths specified in Table 6.6.2.3A.0.3-1. If the measured alternative adjacent channel power is greater than –50dBm then the E-UTRAACLR2 shall be higher than the value specified in Table 6.6.2.3A.0.3-1.

Table 6.6.2.3A.0.3-1: Additional E-UTRAACLR requirement

|  | Channel bandwidth / E-UTRAACLR2 / Measurement bandwidth |
| --- | --- |
|  | 20 MHz |
| E-UTRAACLR2 | 40 dBc |
| E-UTRA channel Measurement bandwidth | NOTE 1 |
| Adjacent channel centre frequency offset [MHz] | +40 / -40 |
| NOTE 1: 18 MHz for E-UTRA channels assigned within 5150-5350 MHz; 19 MHz for E-UTRA channels assigned within 5470-5725 MHz. |  |

The normative reference for this requirement is TS 36.101 [2] subclause 6.6.2.3.1A.

##### 6.6.2.3A.1	Adjacent Channel Leakage power Ratio for CA (intra-band contiguous DL CA and UL CA)

6.6.2.3A.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR) for CA.

6.6.2.3A.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 10 and forward that support intra band contiguous DL CA and UL CA.

6.6.2.3A.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.2.3A.0.

6.6.2.3A.1.4	Test description

6.6.2.3A.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.3A.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.3A.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Low range, High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  | Lowest NRB_agg, Highest NRB_agg (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| ID | PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations(LCRB @ RBstart) |  |  |  |
| 1 | 25 | 25 | N/A for this test | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 2 | 25 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 3 | 25 | 25 |  | QPSK | 50 | P_25@0 | S_25@0 | - | - |
| 4 | 25 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 5 | 25 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 6 | 25 | 25 |  | 16QAM | 50 | P_25@0 | S_25@0 | - | - |
| 7 | 25 | 25 |  | QPSK | 2 | P_1@0 | S_1@24 | - | - |
| 8 | 50 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 9 | 50 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 10 | 50 | 25 |  | QPSK | 75 | P_50@0 | S_25@0 | - | - |
| 11 | 50 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 12 | 50 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 13 | 50 | 25 |  | 16QAM | 75 | P_50@0 | S_25@0 | - | - |
| 14 | 50 | 25 |  | QPSK | 2 | P_1@0 | S_1@24 | - | - |
| 15 | 50 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 16 | 50 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 17 | 50 | 50 |  | QPSK | 100 | P_50@0 | S_50@0 | - | - |
| 18 | 50 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 19 | 50 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 20 | 50 | 50 |  | 16QAM | 100 | P_50@0 | S_50@0 | - | - |
| 21 | 50 | 50 |  | QPSK | 2 | P_1@0 | S_1@49 | - | - |
| 22 | 75 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 23 | 75 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 24 | 75 | 25 |  | QPSK | 100 | P_75@0 | S_25@0 | - | - |
| 25 | 75 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 26 | 75 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 27 | 75 | 25 |  | 16QAM | 100 | P_75@0 | S_25@0 | - | - |
| 28 | 75 | 25 |  | QPSK | 2 | P_1@0 | S_1@24 | - | - |
| 29 | 75 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 30 | 75 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 31 | 75 | 50 |  | QPSK | 125 | P_75@0 | S_50@0 | - | - |
| 32 | 75 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 33 | 75 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 34 | 75 | 50 |  | 16QAM | 125 | P_75@0 | S_50@0 | - | - |
| 35 | 75 | 50 |  | QPSK | 2 | P_1@0 | S_1@49 | - | - |
| 36 | 75 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 37 | 75 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 | - | - |
| 38 | 75 | 75 |  | QPSK | 150 | P_75@0 | S_75@0 | - | - |
| 39 | 75 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 | - | - |
| 40 | 75 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 | - | - |
| 41 | 75 | 75 |  | 16QAM | 150 | P_75@0 | S_75@0 | - | - |
| 42 | 75 | 75 |  | QPSK | 2 | P_1@0 | S_1@74 | - | - |
| 43 | 100 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 44 | 100 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 45 | 100 | 25 |  | QPSK | 125 | P_100@0 | S_25@0 | - | - |
| 46 | 100 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 47 | 100 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 48 | 100 | 25 |  | 16QAM | 125 | P_100@0 | S_25@0 | - | - |
| 49 | 100 | 25 |  | QPSK | 2 | P_1@0 | S_1@24 | - | - |
| 50 | 100 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 51 | 100 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 52 | 100 | 50 |  | QPSK | 150 | P_100@0 | S_50@0 | - | - |
| 53 | 100 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 54 | 100 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 55 | 100 | 50 |  | 16QAM | 150 | P_100@0 | S_50@0 | - | - |
| 56 | 100 | 50 |  | QPSK | 2 | P_1@0 | S_1@49 | - | - |
| 57 | 100 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 58 | 100 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 | - | - |
| 59 | 100 | 75 |  | QPSK | 175 | P_100@0 | S_75@0 | - | - |
| 60 | 100 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 | - | - |
| 61 | 100 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 | - | - |
| 62 | 100 | 75 |  | 16QAM | 175 | P_100@0 | S_75@0 | - | - |
| 63 | 100 | 75 |  | QPSK | 2 | P_1@0 | S_1@74 | - | - |
| 64 | 100 | 100 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| 65 | 100 | 100 |  | QPSK | 100 | P_100@0 | S_0@0 | - | - |
| 66 | 100 | 100 |  | QPSK | 200 | P_100@0 | S_100@0 | - | - |
| 67 | 100 | 100 |  | 16QAM | 18 | P_18@0 | S_0@0 | - | - |
| 68 | 100 | 100 |  | 16QAM | 100 | P_100@0 | S_0@0 | - | - |
| 69 | 100 | 100 |  | 16QAM | 200 | P_100@0 | S_100@0 | - | - |
| 70 | 100 | 100 |  | QPSK | 2 | P_1@0 | S_1@99 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.3A.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.3A.1.4.3.

6.6.2.3A.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.6.2.3A.1.4.3

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.6.2.3A.1.4.1-1 on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control “up” commands in every uplink scheduling information to the UE until the UE transmits at PUMAX level.

6.	Measure the mean power over all component carriers of the UE in the CA configuration of the radio access mode according to the test configuration, which shall meet the requirements described in Table 6.2.3A.1.5-1. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

7.	Measure the rectangular filtered mean power for CA E-UTRA using a rms detector.

8.	Measure the rectangular filtered mean power of the first CA E-UTRA adjacent channel on both lower and upper side of the CA E-UTRA channel using a rms detector, respectively.

9.	Measure the RRC filtered mean power of the first and the second UTRA adjacent channel on both lower and upper side of the CA E-UTRA channel using a rms detector, respectively.

10.	Calculate the ratio of the power between the values measured in step 7 over step 8 for CA E-UTRAACLR.

11.	Calculate the ratio of the power between the values measured in step 7 over step 9 for UTRAACLR1, UTRAACLR2.

6.6.2.3A.1.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

6.6.2.3A.1.5	Test Requirements

6.6.2.3A.1.5.1	Test requirements of UTRA for CA

If the measured UTRA channel power is greater than –50dBm then the measured UTRAACLR1 and UTRAACLR2, derived in step 11, shall be higher than the limits in table 6.6.2.3A.1.5.1-1.

Table 6.6.2.3A.1.5.1-1: UTRA UE ACLR for CA

|  | CA bandwidth class / UTRAACLR1/2 / measurement bandwidth |
| --- | --- |
|  | CA bandwidth class B and C |
| UTRAACLR1 | 32.2 dB |
| Adjacent channel centre frequency offset (in MHz) | + BWChannel_CA /2 + BWUTRA/2 / - BWChannel_CA / 2 - BWUTRA/2 |
| UTRAACLR2 | 35.2 dB |
| Adjacent channel centre frequency offset (in MHz) | + BWChannel_CA /2 + 3*BWUTRA/2 / - BWChannel_CA /2 – 3*BWUTRA/2 |
| CA E-UTRA channel Measurement bandwidth | BWChannel_CA - 2* BWGB |
| UTRA 5MHz channel Measurement bandwidth (Note 1) | 3.84 MHz |
| UTRA 1.6MHz channel measurement bandwidth (Note 2) | 1.28 MHz |
| Note 1: Applicable for E-UTRA FDD co-existence with UTRA FDD in paired spectrum. Note 2: Applicable for E-UTRA TDD co-existence with UTRA TDD in unpaired spectrum. |  |

6.6.2.3A.1.5.2	Test requirements of CA E-UTRA

-	The measured UE mean power in the channel bandwidth as specified in clause 5.4.2A, derived in step 6, shall fulfil requirements in Table 6.2.3A.1.5-1 as appropriate,

and

-	if the measured adjacent channel power is greater than –50 dBm then the measured CA E-UTRAACLR, derived in step 10, shall be higher than the limits in table 6.6.2.3A.1.5.2-1.

Table 6.6.2.3A.1.5.2-1: CA E-UTRA ACLR

|  | CA bandwidth class / CA E-UTRAACLR / Measurement bandwidth |
| --- | --- |
|  | CA bandwidth class B and C |
| CA E-UTRAACLR | 29.2 dB |
| CA E-UTRA channel Measurement bandwidth | BWChannel_CA - 2* BWGB |
| Adjacent channel centre frequency offset (in MHz) | + BWChannel_CA / - BWChannel_CA |

##### 6.6.2.3A.1_1	Adjacent Channel Leakage power Ratio for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM

6.6.2.3A.1_1.1	Test purpose

Same test purpose as defined in clause 6.6.2.3A.1.1.

6.6.2.3A.1_1.2	Test applicability

This test case applies to all types of E-UTRA UE from release 13 and forward that support intra-band contiguous DL CA, UL CA and UL 64QAM.

6.6.2.3A.1_1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.2.3A.0.

6.6.2.3A.1_1.4	Test description

6.6.2.3A.1_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in Table 5.4.2A.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 6.6.2.3A.1_1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.3A.1_1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Low range, High range |  |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  | Lowest NRB_agg, Highest NRB_agg (Note 2) |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  |  | DL Allocation | CC MOD |  | UL Allocation |  |  |  |  |
| ID | PCC NRB | SCCs NRB | PCC & SCC RB allocation |  |  | NRB_allo | PCC & SCC RB allocations(LCRB @ RBstart) |  |  |  |
| 1 | 100 | 25 | N/A | 64QAM |  | 8 | P_8@0 | S_0@0 |  |  |
| 2 | 100 | 25 |  | 64QAM |  | 100 | P_100@0 | S_0@0 |  |  |
| 3 | 100 | 25 |  | 64QAM |  | 125 | P_100@0 | S_25@0 |  |  |
| 4 | 100 | 50 |  | 64QAM |  | 12 | P_12@0 | S_0@0 |  |  |
| 5 | 100 | 50 |  | 64QAM |  | 100 | P_100@0 | S_0@0 |  |  |
| 6 | 100 | 50 |  | 64QAM |  | 150 | P_100@0 | S_50@0 |  |  |
| 7 | 75 | 25 |  | 64QAM |  | 8 | P_8@0 | S_0@0 |  |  |
| 8 | 75 | 25 |  | 64QAM |  | 75 | P_75@0 | S_0@0 |  |  |
| 9 | 75 | 25 |  | 64QAM |  | 100 | P_75@0 | S_25@0 |  |  |
| 10 | 75 | 75 |  | 64QAM |  | 16 | P_16@0 | S_0@0 |  |  |
| 11 | 75 | 75 |  | 64QAM |  | 75 | P_75@0 | S_0@0 |  |  |
| 12 | 75 | 75 |  | 64QAM |  | 150 | P_75@0 | S_75@0 |  |  |
| 13 | 100 | 75 |  | 64QAM |  | 16 | P_16@0 | S_0@0 |  |  |
| 14 | 100 | 75 |  | 64QAM |  | 100 | P_100@0 | S_0@0 |  |  |
| 15 | 100 | 75 |  | 64QAM |  | 175 | P_100@0 | S_75@0 |  |  |
| 16 | 100 | 100 |  | 64QAM |  | 18 | P_18@0 | S_0@0 |  |  |
| 17 | 100 | 100 |  | 64QAM |  | 100 | P_100@0 | S_0@0 |  |  |
| 18 | 100 | 100 |  | 64QAM |  | 200 | P_100@0 | S_100@0 |  |  |
| 19 | 25 | 25 |  | 64QAM |  | 8 | P_8@0 | S_0@0 |  |  |
| 20 | 25 | 25 |  | 64QAM |  | 25 | P_25@0 | S_0@0 |  |  |
| 21 | 25 | 25 |  | 64QAM |  | 50 | P_25@0 | S_25@0 |  |  |
| 22 | 25 | 50 |  | 64QAM |  | 8 | P_8@0 | S_0@0 |  |  |
| 23 | 25 | 50 |  | 64QAM |  | 25 | P_25@0 | S_0@0 |  |  |
| 24 | 25 | 50 |  | 64QAM |  | 75 | P_25@0 | S_50@0 |  |  |
| 25 | 50 | 50 |  | 64QAM |  | 12 | P_12@0 | S_0@0 |  |  |
| 26 | 50 | 50 |  | 64QAM |  | 50 | P_50@0 | S_0@0 |  |  |
| 27 | 50 | 50 |  | 64QAM |  | 100 | P_50@0 | S_50@0 |  |  |
| 28 | 50 | 100 |  | 64QAM |  | 12 | P_12@0 | S_0@0 |  |  |
| 29 | 50 | 100 |  | 64QAM |  | 50 | P_50@0 | S_0@0 |  |  |
| 30 | 50 | 100 |  | 64QAM |  | 150 | P_50@0 | S_100@0 |  |  |
| 31 | 50 | 25 |  | 64QAM |  | 8 | P_8@0 | S_0@0 |  |  |
| 32 | 50 | 25 |  | 64QAM |  | 25 | P_50@0 | S_0@0 |  |  |
| 33 | 50 | 25 |  | 64QAM |  | 75 | P_50@0 | S_25@0 |  |  |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |  |  |

6.6.2.3A.1_1.4.2	Test procedure

Same test procedure as defined in clause 6.6.2.3A.1.4.2 with following exceptions:

-	Instead of Table 6.2.3A.1.5-1  use Table 6.2.3A.1_1.5-1.

6.6.2.3A.1_1.4.3	Message contents

Same message contents as defined in clause 6.6.2.3A.1.4.3.

6.6.2.3A.1_1.5	Test requirement

Same test requirement as defined in clause 6.6.2.3A.1.5 with following exceptions:

-	Instead of Table 6.2.3A.1.5-1  use Table 6.2.3A.1_1.5-1.

##### 6.6.2.3A.1_2	Adjacent Channel Leakage power Ratio for CA (intra-band contiguous DL CA and UL CA) for UL 256QAM

6.6.2.3A.1_2.1	Test purpose

Same test purpose as defined in clause 6.6.2.3A.1.1.

6.6.2.3A.1_2.2	Test applicability

This test case applies to all types of E-UTRA UE from release 14 and forward that support intra-band contiguous DL CA, UL CA and UL 256QAM.

6.6.2.3A.1_2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.2.3A.0.

6.6.2.3A.1_2.4	Test description

6.6.2.3A.1_2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in Table 5.4.2A.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 6.6.2.3A.1_2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.3A.1_2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Low and High range |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  |  | DL Allocation | CC MOD | UL Allocation |  |  |
| ID | PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_ alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |
| 1 | 25 | 50 | N/A | 256QAM | 25 | P_25@0 | S_0@0 |
| 2 | 25 | 50 |  | 256QAM | 75 | P_25@0 | S_50@0 |
| 3 | 50 | 50 |  | 256QAM | 50 | P_50@0 | S_0@0 |
| 4 | 50 | 50 |  | 256QAM | 100 | P_50@0 | S_50@0 |
| 5 | 50 | 100 |  | 256QAM | 50 | P_50@0 | S_0@0 |
| 6 | 50 | 100 |  | 256QAM | 150 | P_50@0 | S_100@0 |
| 7 | 75 | 50 |  | 256QAM | 75 | P_75@0 | S_0@0 |
| 8 | 75 | 50 |  | 256QAM | 125 | P_75@0 | S_50@0 |
| 9 | 75 | 75 |  | 256QAM | 75 | P_75@0 | S_0@0 |
| 10 | 75 | 75 |  | 256QAM | 150 | P_75@0 | S_75@0 |
| 11 | 100 | 25 |  | 256QAM | 100 | P_100@0 | S_0@0 |
| 12 | 100 | 25 |  | 256QAM | 125 | P_100@0 | S_25@0 |
| 13 | 100 | 50 |  | 256QAM | 100 | P_100@0 | S_0@0 |
| 14 | 100 | 50 |  | 256QAM | 150 | P_100@0 | S_50@0 |
| 15 | 100 | 75 |  | 256QAM | 100 | P_100@0 | S_0@0 |
| 16 | 100 | 75 |  | 256QAM | 175 | P_100@0 | S_75@0 |
| 17 | 100 | 100 |  | 256QAM | 100 | P_100@0 | S_0@0 |
| 18 | 100 | 100 |  | 256QAM | 200 | P_100@0 | S_100@0 |
| 19 | 50 | 25 |  | 256QAM | 50 | P_50@0 | S_0@0 |
| 20 | 50 | 25 |  | 256QAM | 75 | P_50@0 | S_25@0 |
| NOTE 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. NOTE 2: If UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |

6.6.2.3A.1_2.4.2	Test procedure

Same test procedure as defined in clause 6.6.2.3A.1.4.2 with following exceptions:

-	In step 2, instead of clause 6.6.2.3A.1.4.3  use clause 6.6.2.3A.1_2.4.3.

-	In step 4, instead of Table 6.6.2.3A.1.4.1-1  use Table 6.6.2.3A.1_2.4.1-1.

-	In step 6, instead of Table 6.2.3A.1.5-1  use Table 6.2.3A.1_2.5-1.

6.6.2.3A.1_2.4.3	Message contents

Same message contents as defined in clause 6.6.2.3A.1.4.3.

6.6.2.3A.1_2.5	Test requirement

Same test requirement as defined in clause 6.6.2.3A.1.5 with following exception:

-	Instead of Table 6.2.3A.1.5-1  use Table 6.2.3A.1_2.5-1.

##### 6.6.2.3A.1_3	Adjacent Channel Leakage power Ratio for CA (intra-band contiguous DL CA and UL CA) for HPUE

6.6.2.3A.1_3.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR) for CA.

6.6.2.3A.1_3.2	Test applicability

This test applies to all types of E-UTRA UE Power Class 2 release 10 and forward that support intra band contiguous DL CA and UL CA.

6.6.2.3A.1_3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.2.3A.0.

6.6.2.3A.1_3.4	Test description

6.6.2.3A.1_3.4.1	Initial condition

Use 6.6.2.3A.1.4.1 except replace Table 6.6.2.3A.1.4.1-1 with Table 6.6.2.3A.1_3.4.1-1.

Table 6.6.2.3A.1_3.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Low and High range |  |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| ID |  | PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_ alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 100 | 25 |  | QPSK | 6 | P_6@0 | S_0@0 |  |  |  |
| 2 | 100 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 |  |  |  |
| 3 | 100 | 25 |  | QPSK | 125 | P_100@0 | S_25@0 |  |  |  |
| 4 | 100 | 25 |  | 16QAM | 6 | P_6@0 | S_0@0 |  |  |  |
| 5 | 100 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 |  |  |  |
| 6 | 100 | 25 |  | 16QAM | 125 | P_100@0 | S_25@0 |  |  |  |
| 7 | 100 | 25 |  | QPSK | 2 | P_1@0 | S_1@24 |  |  |  |
| 8 | 75 | 75 |  | QPSK | 6 | P_6@0 | S_0@0 |  |  |  |
| 9 | 75 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 |  |  |  |
| 10 | 75 | 75 |  | QPSK | 150 | P_75@0 | S_75@0 |  |  |  |
| 11 | 75 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 |  |  |  |
| 12 | 75 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 |  |  |  |
| 13 | 75 | 75 |  | 16QAM | 150 | P_75@0 | S_75@0 |  |  |  |
| 14 | 75 | 75 |  | QPSK | 2 | P_1@0 | S_1@74 |  |  |  |
| 15 | 100 | 50 |  | QPSK | 6 | P_6@0 | S_0@0 |  |  |  |
| 16 | 100 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 |  |  |  |
| 17 | 100 | 50 |  | QPSK | 150 | P_100@0 | S_50@0 |  |  |  |
| 18 | 100 | 50 |  | 16QAM | 6 | P_6@0 | S_0@0 |  |  |  |
| 19 | 100 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 |  |  |  |
| 20 | 100 | 50 |  | 16QAM | 150 | P_100@0 | S_50@0 |  |  |  |
| 21 | 100 | 50 |  | QPSK | 2 | P_1@0 | S_1@49 |  |  |  |
| 22 | 100 | 75 |  | QPSK | 6 | P_6@0 | S_0@0 |  |  |  |
| 23 | 100 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 |  |  |  |
| 24 | 100 | 75 |  | QPSK | 175 | P_100@0 | S_75@0 |  |  |  |
| 25 | 100 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 |  |  |  |
| 26 | 100 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 |  |  |  |
| 27 | 100 | 75 |  | 16QAM | 175 | P_100@0 | S_75@0 |  |  |  |
| 28 | 100 | 75 |  | QPSK | 2 | P_1@0 | S_1@74 |  |  |  |
| 29 | 100 | 100 |  | QPSK | 6 | P_6@0 | S_0@0 |  |  |  |
| 30 | 100 | 100 |  | QPSK | 100 | P_100@0 | S_0@0 |  |  |  |
| 31 | 100 | 100 |  | QPSK | 200 | P_100@0 | S_100@0 |  |  |  |
| 32 | 100 | 100 |  | 16QAM | 18 | P_18@0 | S_0@0 |  |  |  |
| 33 | 100 | 100 |  | 16QAM | 100 | P_100@0 | S_0@0 |  |  |  |
| 34 | 100 | 100 |  | 16QAM | 200 | P_100@0 | S_100@0 |  |  |  |
| 35 | 100 | 100 |  | QPSK | 2 | P_1@0 | S_1@99 |  |  |  |
| 36 | 75 | 50 |  | QPSK | 6 | P_6@0 | S_0@0 |  |  |  |
| 37 | 75 | 50 |  | QPSK | 25 | P_25@0 | S_0@0 |  |  |  |
| 38 | 75 | 50 |  | QPSK | 125 | P_75@0 | S_50@0 |  |  |  |
| 39 | 75 | 50 |  | 16QAM | 6 | P_6@0 | S_0@0 |  |  |  |
| 40 | 75 | 50 |  | 16QAM | 25 | P_25@0 | S_0@0 |  |  |  |
| 41 | 75 | 50 |  | 16QAM | 125 | P_75@0 | S_50@0 |  |  |  |
| 42 | 75 | 50 |  | QPSK | 2 | P_1@0 | S_1@24 |  |  |  |
| 43 | 100 | 25 |  | 64QAM | 8 | P_8@0 | S_0@0 |  |  |  |
| 44 | 100 | 25 |  | 64QAM | 125 | P_100@0 | S_25@0 |  |  |  |
| 45 | 75 | 75 |  | 64QAM | 16 | P_16@0 | S_0@0 |  |  |  |
| 46 | 75 | 75 |  | 64QAM | 150 | P_75@0 | S_75@0 |  |  |  |
| 47 | 100 | 50 |  | 64QAM | 12 | P_12@0 | S_0@0 |  |  |  |
| 48 | 100 | 50 |  | 64QAM | 150 | P_100@0 | S_50@0 |  |  |  |
| 49 | 100 | 75 |  | 64QAM | 16 | P_16@0 | S_0@0 |  |  |  |
| 50 | 100 | 75 |  | 64QAM | 175 | P_100@0 | S_75@0 |  |  |  |
| 51 | 100 | 100 |  | 64QAM | 18 | P_18@0 | S_0@0 |  |  |  |
| 52 | 100 | 100 |  | 64QAM | 200 | P_100@0 | S_100@0 |  |  |  |
| 53 | 75 | 50 |  | 64QAM | 12 | P_12@0 | S_0@0 |  |  |  |
| 54 | 75 | 50 |  | 64QAM | 125 | P_75@0 | S_50@0 |  |  |  |
| NOTE 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. NOTE 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |  |  |

6.6.2.3A.1_3.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.6.2.3A.1.4.3

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.6.2.3A.1.4.1-1 on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control “up” commands in every uplink scheduling information to the UE until the UE transmits at PUMAX level.

6.	Measure the mean power over all component carriers of the UE in the CA configuration of the radio access mode according to the test configuration, which shall meet the requirements described in Table 6.2.3A.1_3.5-1. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

7.	Measure the rectangular filtered mean power for CA E-UTRA using a rms detector.

8.	Measure the rectangular filtered mean power of the first CA E-UTRA adjacent channel on both lower and upper side of the CA E-UTRA channel using a rms detector, respectively.

9.	Measure the RRC filtered mean power of the first and the second UTRA adjacent channel on both lower and upper side of the CA E-UTRA channel using a rms detector, respectively.

10.	Calculate the ratio of the power between the values measured in step 7 over step 8 for CA E-UTRAACLR.

11.	Calculate the ratio of the power between the values measured in step 7 over step 9 for UTRAACLR1, UTRAACLR2.

6.6.2.3A.1_3.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

6.6.2.3A.1_3.5	Test Requirements

6.6.2.3A.1_3.5.1	Test requirements of UTRA for CA

If the measured UTRA channel power is greater than –50dBm then the measured UTRAACLR1 and UTRAACLR2, derived in step 11, shall be higher than the limits in table 6.6.2.3A.1.5.1-1.

Table 6.6.2.3A.1.5.1-1: UTRA UE ACLR for CA

|  | CA bandwidth class / UTRAACLR1/2 / measurement bandwidth |
| --- | --- |
|  | CA bandwidth class B and C |
| UTRAACLR1 | 32.2 dB |
| Adjacent channel centre frequency offset (in MHz) | + BWChannel_CA /2 + BWUTRA/2 / - BWChannel_CA / 2 - BWUTRA/2 |
| UTRAACLR2 | 35.2 dB |
| Adjacent channel centre frequency offset (in MHz) | + BWChannel_CA /2 + 3*BWUTRA/2 / - BWChannel_CA /2 – 3*BWUTRA/2 |
| CA E-UTRA channel Measurement bandwidth | BWChannel_CA - 2* BWGB |
| UTRA 5MHz channel Measurement bandwidth (Note 1) | 3.84 MHz |
| UTRA 1.6MHz channel measurement bandwidth (Note 2) | 1.28 MHz |
| Note 1: Applicable for E-UTRA FDD co-existence with UTRA FDD in paired spectrum. Note 2: Applicable for E-UTRA TDD co-existence with UTRA TDD in unpaired spectrum. |  |

6.6.2.3A.1_3.5.2	Test requirements of CA E-UTRA

-	The measured UE mean power in the channel bandwidth as specified in clause 5.4.2A, derived in step 6, shall fulfil requirements in Table 6.2.3A.1.5-1 as appropriate,

and

-	if the measured adjacent channel power is greater than –50 dBm then the measured CA E-UTRAACLR, derived in step 10, shall be higher than the limits in table 6.6.2.3A.1.5.2-1.

Table 6.6.2.3A.1_3.5.2-1: CA E-UTRA ACLR

|  | CA bandwidth class / CA E-UTRAACLR / Measurement bandwidth |
| --- | --- |
|  | CA bandwidth class B and C |
| CA E-UTRAACLR | 29.2 dB |
| CA E-UTRA channel Measurement bandwidth | BWChannel_CA - 2* BWGB |
| Adjacent channel centre frequency offset (in MHz) | + BWChannel_CA / - BWChannel_CA |

##### 6.6.2.3A.2	Adjacent Channel Leakage power Ratio for CA (inter-band DL CA and UL CA)

6.6.2.3A.2.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR) for CA.

6.6.2.3A.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 11 and forward that support inter-band DL CA and UL CA. The requirements also apply to all types of E-UTRA UE release 14 and forward that support uplink LAA.

6.6.2.3A.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.2.3A.0.

6.6.2.3A.2.4	Test description

6.6.2.3A.2.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in Table 5.4.2A.1-2 for CA configurations with two bands or Table 5.4.2A.1-2a for CA configurations with three bands. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.3A.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.3A.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC (Note 3) |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |  |
|  | PCC | SCC | PCC & SCC RB allocation | PCC | SCC | NRB_alloc | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |  |
| 1 | 6 | 25 | N/A for this test | QPSK | QPSK | 13 | 5 | 8 |
| 2 | 6 | 25 |  | QPSK | QPSK | 31 | 6 | 25 |
| 3 | 6 | 25 |  | 16QAM | 16QAM | 13 | 5 | 8 |
| 4 | 6 | 25 |  | 16QAM | 16QAM | 31 | 6 | 25 |
| 5 | 6 | 25 |  | QPSK | 16QAM | 30 | 5 | 25 |
| 6 | 6 | 50 |  | QPSK | QPSK | 17 | 5 | 12 |
| 7 | 6 | 50 |  | QPSK | QPSK | 56 | 6 | 50 |
| 8 | 6 | 50 |  | 16QAM | 16QAM | 17 | 5 | 12 |
| 9 | 6 | 50 |  | 16QAM | 16QAM | 56 | 6 | 50 |
| 10 | 6 | 50 |  | QPSK | 16QAM | 55 | 5 | 50 |
| 11 | 25 | 15 |  | QPSK | QPSK | 12 | 8 | 4 |
| 12 | 25 | 15 |  | QPSK | QPSK | 40 | 25 | 15 |
| 13 | 25 | 15 |  | 16QAM | 16QAM | 12 | 8 | 4 |
| 14 | 25 | 15 |  | 16QAM | 16QAM | 40 | 25 | 15 |
| 15 | 25 | 15 |  | QPSK | 16QAM | 23 | 8 | 15 |
| 16 | 25 | 25 |  | QPSK | QPSK | 16 | 8 | 8 |
| 17 | 25 | 25 |  | QPSK | QPSK | 50 | 25 | 25 |
| 18 | 25 | 25 |  | 16QAM | 16QAM | 16 | 8 | 8 |
| 19 | 25 | 25 |  | 16QAM | 16QAM | 50 | 25 | 25 |
| 20 | 25 | 25 |  | QPSK | 16QAM | 33 | 8 | 25 |
| 21 | 25 | 50 |  | QPSK | QPSK | 16 | 4 | 12 |
| 22 | 25 | 50 |  | QPSK | QPSK | 65 | 15 | 50 |
| 23 | 25 | 50 |  | 16QAM | 16QAM | 16 | 4 | 12 |
| 24 | 25 | 50 |  | 16QAM | 16QAM | 65 | 15 | 50 |
| 25 | 25 | 50 |  | QPSK | 16QAM | 54 | 4 | 50 |
| 26 | 50 | 25 |  | QPSK | QPSK | 20 | 12 | 8 |
| 27 | 50 | 25 |  | QPSK | QPSK | 75 | 50 | 25 |
| 28 | 50 | 25 |  | 16QAM | 16QAM | 20 | 12 | 8 |
| 29 | 50 | 25 |  | 16QAM | 16QAM | 75 | 50 | 25 |
| 30 | 50 | 25 |  | QPSK | 16QAM | 37 | 12 | 25 |
| 31 | 50 | 50 |  | QPSK | QPSK | 24 | 12 | 12 |
| 32 | 50 | 50 |  | QPSK | QPSK | 100 | 50 | 50 |
| 33 | 50 | 50 |  | 16QAM | 16QAM | 24 | 12 | 12 |
| 34 | 50 | 50 |  | 16QAM | 16QAM | 100 | 50 | 50 |
| 35 | 50 | 50 |  | QPSK | 16QAM | 62 | 12 | 50 |
| 36 | 50 | 100 |  | QPSK | QPSK | 30 | 12 | 18 |
| 37 | 50 | 100 |  | QPSK | QPSK | 150 | 50 | 100 |
| 38 | 50 | 100 |  | 16QAM | 16QAM | 30 | 12 | 18 |
| 39 | 50 | 100 |  | 16QAM | 16QAM | 150 | 50 | 100 |
| 40 | 50 | 100 |  | QPSK | 16QAM | 112 | 12 | 100 |
| 41 | 75 | 75 |  | QPSK | QPSK | 32 | 16 | 16 |
| 42 | 75 | 75 |  | QPSK | QPSK | 150 | 75 | 75 |
| 43 | 75 | 75 |  | 16QAM | 16QAM | 32 | 16 | 16 |
| 44 | 75 | 75 |  | 16QAM | 16QAM | 150 | 75 | 75 |
| 45 | 75 | 75 |  | QPSK | 16QAM | 91 | 16 | 75 |
| 46 | 100 | 50 |  | QPSK | QPSK | 30 | 18 | 12 |
| 47 | 100 | 50 |  | QPSK | QPSK | 150 | 100 | 50 |
| 48 | 100 | 50 |  | 16QAM | 16QAM | 30 | 18 | 12 |
| 49 | 100 | 50 |  | 16QAM | 16QAM | 150 | 100 | 50 |
| 50 | 100 | 50 |  | QPSK | 16QAM | 68 | 18 | 50 |
| 51 | 100 | 75 |  | QPSK | QPSK | 34 | 18 | 16 |
| 52 | 100 | 75 |  | QPSK | QPSK | 175 | 100 | 75 |
| 53 | 100 | 75 |  | 16QAM | 16QAM | 34 | 18 | 16 |
| 54 | 100 | 75 |  | 16QAM | 16QAM | 175 | 100 | 75 |
| 55 | 100 | 75 |  | QPSK | 16QAM | 93 | 18 | 75 |
| 56 | 100 | 100 |  | QPSK | QPSK | 36 | 18 | 18 |
| 57 | 100 | 100 |  | QPSK | QPSK | 200 | 100 | 100 |
| 58 | 100 | 100 |  | 16QAM | 16QAM | 36 | 18 | 18 |
| 59 | 100 | 100 |  | 16QAM | 16QAM | 200 | 100 | 100 |
| 60 | 100 | 100 |  | QPSK | 16QAM | 118 | 18 | 100 |
| 61 | 15 | 25 |  | QPSK | QPSK | 12 | 4 | 8 |
| 62 | 15 | 25 |  | QPSK | QPSK | 40 | 15 | 25 |
| 63 | 15 | 25 |  | 16QAM | 16QAM | 12 | 4 | 8 |
| 64 | 15 | 25 |  | 16QAM | 16QAM | 40 | 15 | 25 |
| 65 | 15 | 25 |  | QPSK | 16QAM | 29 | 4 | 25 |
| 66 | 25 | 6 |  | QPSK | QPSK | 13 | 8 | 5 |
| 67 | 25 | 6 |  | QPSK | QPSK | 31 | 25 | 6 |
| 68 | 25 | 6 |  | 16QAM | 16QAM | 13 | 8 | 5 |
| 69 | 25 | 6 |  | 16QAM | 16QAM | 31 | 25 | 6 |
| 70 | 25 | 6 |  | QPSK | 16QAM | 14 | 8 | 6 |
| 71 | 25 | 75 |  | QPSK | QPSK | 24 | 8 | 16 |
| 72 | 25 | 75 |  | QPSK | QPSK | 100 | 25 | 75 |
| 73 | 25 | 75 |  | 16QAM | 16QAM | 24 | 8 | 16 |
| 74 | 25 | 75 |  | 16QAM | 16QAM | 100 | 25 | 75 |
| 75 | 25 | 75 |  | QPSK | 16QAM | 83 | 8 | 75 |
| 76 | 75 | 25 |  | QPSK | QPSK | 24 | 16 | 8 |
| 77 | 75 | 25 |  | QPSK | QPSK | 100 | 75 | 25 |
| 78 | 75 | 25 |  | 16QAM | 16QAM | 24 | 16 | 8 |
| 79 | 75 | 25 |  | 16QAM | 16QAM | 100 | 75 | 25 |
| 80 | 75 | 25 |  | QPSK | 16QAM | 41 | 16 | 25 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg. only the combination with the highest NRB_PCC is tested. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBmax-LCRB+1. |  |  |  |  |  |  |  |  |

Table 6.6.2.3A.2.4.1-2: Test Configuration Table for UL LAA

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC (Note 2) and SCC (Note 3) |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest NRB_agg Highest NRB_agg |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |  |
|  | PCC | SCC | PCC & SCC RB allocation | PCC | SCC | NRB_alloc | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |  |
| 1 | 100 | 100 | N/A for this test | QPSK | QPSK | 11 | 1 | L=1 |
| 2 | 100 | 100 |  | QPSK | QPSK | 108 | 18 | L=9 |
| 3 | 100 | 100 |  | 16QAM | 16QAM | 11 | 1 | L=1 |
| 4 | 100 | 100 |  | 16QAM | 16QAM | 108 | 18 | L=9 |
| 5 | 100 | 100 |  | QPSK | QPSK | 200 | 100 | L=10 |
| 6 | 100 | 100 |  | 16QAM | 16QAM | 200 | 100 | L=10 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBmax-LCRB+1. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=N-L, with N defined in Clause 8.1.4 of [10]. Note 4: Applicable only for an LAA Scell configured in Band 46. |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.3A.2.4.1-1 or Table 6.6.2.3A.2.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.3A.2.4.3.

6.6.2.3A.2.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.6.2.3A.2.4.3

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 or DCI format 0A for LAA SCC for C_RNTI to schedule the UL RMC according to Table 6.6.2.3A.2.4.1-1 or Table 6.6.2.3A.2.4.1-2 on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control “up” commands in every uplink scheduling information to the UE until the UE transmits at PUMAX level.

6.	Measure the mean power of the component carrier of the UE in the first band of the CA configuration of the radio access mode according to the test configuration, which shall meet the requirements described in Table 6.2.3A.2.5-1 or Table 6.2.3A.2.5-2. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

7.	Measure the rectangular filtered mean power for CA E-UTRA in the first band of the CA configuration using a rms detector.

8.	Measure the rectangular filtered mean power of the first CA E-UTRA adjacent channel on both lower and upper side of the CA E-UTRA channel in the first band of the CA configuration using a rms detector, respectively.

9.	Measure the RRC filtered mean power of the first and the second UTRA adjacent channel on both lower and upper side of the CA E-UTRA channel in the first band of the CA configuration using a rms detector, respectively.

10.	Calculate the ratio of the power between the values measured in step 7 over step 8 for CA E-UTRAACLR.

11.	Calculate the ratio of the power between the values measured in step 7 over step 9 for UTRAACLR1, UTRAACLR2.

12. Repeat steps 6 to 11 for the second band of the CA configuration.

6.6.2.3A.2.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

For SCC in the case of uplink LAA, message contents are according to TS 36.508[7] subclause4.6 with the following exceptions specified in Tables 6.6.2.3A.1.2.4.3-1, 6.6.2.3A.1.2.4.3-2 and 6.6.2.3A.1.2.4.3-3.

Table 6.6.2.3A.1.2.4.3-1: LogicalChannelConfig-DRB (Initial Condition, step 14, Table 4.5.2.3-1)

| Derivation Path: 36.508 Table 4.8.2.1.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LogicalChannelConfig-DRB ::= SEQUENCE { |  |  |  |  |
| ul-SpecificParameters SEQUENCE { |  |  |  |  |
| laa-UL-Allowed-r14 |  | true |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 6.6.2.3A.1.2.4.3-2: PhysicalConfigDedicatedSCell (Step 2, test procedure)

| Derivation Path: 36.508 Table 4.6.3-6A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicatedSCell-r10-DEFAULT ::= SEQUENCE { |  |  |  |
| laa-SCellConfiguration-v1430 ::= SEQUENCE { |  |  |  |
|  | NULL |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.2.3A.1.2.4.3-3: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_29" on SCC

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 29 (NS_29) |  |  |

6.6.2.3A.2.5	Test Requirements

6.6.2.3A.2.5.1	Test requirements of UTRA for CA

If the measured UTRA channel power is greater than –50dBm then the measured UTRAACLR1 and UTRAACLR2, derived in step 11, shall be higher than the limits in table 6.6.2.3A.2.5.1-1.

Table 6.6.2.3A.2.5.1-1: UTRA UE ACLR for CA

|  | Channel bandwidth / UTRAACLR1/2 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| UTRAACLR1 | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB |
| Adjacent channel centre frequency offset (in MHz) | 0.7+BWUTRA/2 / -0.7-BWUTRA/2 | 1.5+BWUTRA/2 / -1.5-BWUTRA/2 | 2.5+BWUTRA/2 / -2.5-BWUTRA/2 | 5+BWUTRA/2 / -5-BWUTRA/2 | 7.5+BWUTRA/2 / -7.5-BWUTRA/2 | 10+BWUTRA/2 / -10-BWUTRA/2 |
| UTRAACLR2 | - | - | 35.2 dB | 35.2 dB | 35.2 dB | 35.2 dB |
| Adjacent channel centre frequency offset (in MHz) | - | - | 2.5+3*BWUTRA/2 / -2.5-3*BWUTRA/2 | 5+3*BWUTRA/2 / -5-3*BWUTRA/2 | 7.5+3*BWUTRA/2 / -7.5-3*BWUTRA/2 | 10+3*BWUTRA/2 / -10-3*BWUTRA/2 |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UTRA 5MHz channel Measurement bandwidth1 | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz |
| UTRA 1.6MHz channel measurement bandwidth2 | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz |
| Note 1: Applicable for E-UTRA FDD co-existence with UTRA FDD in paired spectrum. Note 2: Applicable for E-UTRA TDD co-existence with UTRA TDD in unpaired spectrum. Note 3: BWUTRA for UTRA FDD is 5MHz and for UTRA TDD is 1.6MHz. |  |  |  |  |  |  |

6.6.2.3A.2.5.2	Test requirements of CA E-UTRA

-	The measured UE mean power in the channel bandwidth as specified in clause 5.4.2A, derived in step 6, shall fulfil requirements in Table 6.2.3A.2.5-1 as appropriate,

and

-	if the measured adjacent channel power is greater than –50 dBm then the measured CA E-UTRAACLR, derived in step 10, shall be higher than the limits in table 6.6.2.3A.2.5.2-1.

Table 6.6.2.3A.2.5.2-1: CA E-UTRA ACLR

|  | Channel bandwidth / E-UTRAACLR1 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UE channel | +1.4 MHz or -1.4 MHz | +3 MHz or -3 MHz | +5MHz or -5MHz | +10MHz or -10MHz | +15MHz or -15MHz | +20MHz or -20MHz |

6.6.2.3A.2.5.3	Test requirements of E-UTRA (network signalled value “NS_29”)

When "NS_29" is indicated in the cell, the UE emission shall meet the additional requirements specified in Table 6.6.2.3A.2.5.3-1 for E-UTRA channels assigned within the frequency ranges 5150-5350 MHz and 5470-5725 MHz.

-	The measured UE mean power in the channel bandwidth as specified in clause 5.4.2A, derived in step 6, shall fulfil requirements in Table 6.2.3A.2.5-1 as appropriate,

and

-	if the measured adjacent channel power is greater than –50 dBm then the measured CA E-UTRAACLR2, derived in step 10, shall be higher than the limits in table 6.6.2.3A.2.5.3-1.

Table 6.6.2.3A.2.5.3-1: Additional E-UTRAACLR requirement

|  | Channel bandwidth / E-UTRAACLR2 / Measurement bandwidth |
| --- | --- |
|  | 20 MHz |
| E-UTRAACLR2 | 38.7 dBc |
| E-UTRA channel Measurement bandwidth | NOTE 1 |
| Adjacent channel centre frequency offset [MHz] | +40 / -40 |
| NOTE 1: 18 MHz for E-UTRA channels assigned within 5150-5350 MHz; 19 MHz for E-UTRA channels assigned within 5470-5725 MHz. |  |

##### 6.6.2.3A.2_1	Adjacent Channel Leakage power Ratio for CA (inter-band DL CA and UL CA) for UL 64QAM

6.6.2.3A.2_1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR) for CA.

6.6.2.3A.2_1.2	Test applicability

This test case applies to all types of E-UTRA UE release 13 and forward that support inter band DL CA and UL CA and UL 64QAM.

6.6.2.3A.2_1.3	Minimum conformance requirements

The minimum requirements are the same as in section 6.6.2.3A.2.3.

6.6.2.3A.2_1.4	Test description

6.6.2.3A.2_1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.3A.2_1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.3A.2_1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC (Note 3) |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |  |
|  | PCC | SCC | PCC & SCC RB allocation | PCC | SCC | NRB_alloc | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |  |
| 1 | 6 | 25 | N/A for this test | 64QAM | 64QAM | 13 | 5 | 8 |
| 2 | 6 | 25 |  | 64QAM | 64QAM | 31 | 6 | 25 |
| 3 | 6 | 50 |  | 64QAM | 64QAM | 17 | 5 | 12 |
| 4 | 6 | 50 |  | 64QAM | 64QAM | 56 | 6 | 50 |
| 5 | 25 | 15 |  | 64QAM | 64QAM | 12 | 8 | 4 |
| 6 | 25 | 15 |  | 64QAM | 64QAM | 40 | 25 | 15 |
| 7 | 25 | 25 |  | 64QAM | 64QAM | 16 | 8 | 8 |
| 8 | 25 | 25 |  | 64QAM | 64QAM | 50 | 25 | 25 |
| 9 | 25 | 50 |  | 64QAM | 64QAM | 16 | 4 | 12 |
| 10 | 25 | 50 |  | 64QAM | 64QAM | 65 | 15 | 50 |
| 11 | 50 | 25 |  | 64QAM | 64QAM | 20 | 12 | 8 |
| 12 | 50 | 25 |  | 64QAM | 64QAM | 75 | 50 | 25 |
| 13 | 50 | 50 |  | 64QAM | 64QAM | 24 | 12 | 12 |
| 14 | 50 | 50 |  | 64QAM | 64QAM | 100 | 50 | 50 |
| 15 | 50 | 100 |  | 64QAM | 64QAM | 30 | 12 | 18 |
| 16 | 50 | 100 |  | 64QAM | 64QAM | 150 | 50 | 100 |
| 17 | 75 | 75 |  | 64QAM | 64QAM | 32 | 16 | 16 |
| 18 | 75 | 75 |  | 64QAM | 64QAM | 150 | 75 | 75 |
| 19 | 100 | 50 |  | 64QAM | 64QAM | 30 | 18 | 12 |
| 20 | 100 | 50 |  | 64QAM | 64QAM | 150 | 100 | 50 |
| 21 | 100 | 75 |  | 64QAM | 64QAM | 34 | 18 | 16 |
| 22 | 100 | 75 |  | 64QAM | 64QAM | 175 | 100 | 75 |
| 23 | 100 | 100 |  | 64QAM | 64QAM | 36 | 18 | 18 |
| 24 | 100 | 100 |  | 64QAM | 64QAM | 200 | 100 | 100 |
| 25 | 15 | 25 |  | 64QAM | 64QAM | 12 | 4 | 8 |
| 26 | 15 | 25 |  | 64QAM | 64QAM | 40 | 15 | 25 |
| 27 | 25 | 6 |  | 64QAM | 64QAM | 13 | 8 | 5 |
| 28 | 25 | 6 |  | 64QAM | 64QAM | 31 | 25 | 6 |
| 29 | 25 | 75 |  | 64QAM | 64QAM | 24 | 8 | 16 |
| 30 | 25 | 75 |  | 64QAM | 64QAM | 100 | 25 | 75 |
| 31 | 75 | 25 |  | 64QAM | 64QAM | 24 | 16 | 8 |
| 32 | 75 | 25 |  | 64QAM | 64QAM | 100 | 75 | 25 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg. only the first of those is tested. according to the order on the Test Configuration Table list. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBmax-LCRB+1. |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.3A.2_1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.3A.1.4.3.

6.6.2.3A.2_1.4.2	Test procedure

Same test procedure as in clause 6.6.2.3A.2.4.2.

6.6.2.3A.2_1.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

6.6.2.3A.2_1.5	Test Requirements

Same test requirements as in clause 6.6.2.3A.2.5.

##### 6.6.2.3A.2_2	Adjacent Channel Leakage power Ratio for CA (inter-band DL CA and UL CA) for UL 256QAM

6.6.2.3A.2_2.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR) for CA.

6.6.2.3A.2_2.2	Test applicability

This test case applies to all types of E-UTRA UE release 14 and forward that support inter band DL CA and UL CA and UL 256QAM.

6.6.2.3A.2_2.3	Minimum conformance requirements

The minimum requirements are the same as in section 6.6.2.3A.2.3.

6.6.2.3A.2_2.4	Test description

6.6.2.3A.2_2.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.3A.2_2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.3A.2_2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  |  | Low and High range for both PCC and SCC |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation | CC MOD |  | UL Allocation |  |  |
|  | PCC | SCC | PCC & SCC RB allocation | PCC | SCC | NRB_alloc | PCC & SCC RB allocations |  |
|  | NRB | NRB |  |  |  |  |  |  |
| 1 | 6 | 25 | N/A for this | 256QAM | 256QAM | 31 | 6 | 25 |
| 2 | 6 | 50 | Test | 256QAM | 256QAM | 56 | 6 | 50 |
| 3 | 25 | 15 |  | 256QAM | 256QAM | 40 | 25 | 15 |
| 4 | 25 | 25 |  | 256QAM | 256QAM | 50 | 25 | 25 |
| 5 | 25 | 50 |  | 256QAM | 256QAM | 75 | 25 | 50 |
| 6 | 50 | 25 |  | 256QAM | 256QAM | 75 | 50 | 25 |
| 7 | 50 | 50 |  | 256QAM | 256QAM | 100 | 50 | 50 |
| 8 | 50 | 100 |  | 256QAM | 256QAM | 150 | 50 | 100 |
| 9 | 75 | 75 |  | 256QAM | 256QAM | 150 | 75 | 75 |
| 10 | 100 | 50 |  | 256QAM | 256QAM | 150 | 100 | 50 |
| 11 | 100 | 75 |  | 256QAM | 256QAM | 175 | 100 | 75 |
| 12 | 100 | 100 |  | 256QAM | 256QAM | 200 | 100 | 100 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.3A.2_2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.3A.2_2.4.3.

6.6.2.3A.2_2.4.2	Test procedure

Same test procedure as in clause 6.6.2.3A.2.4.2 with following exceptions:

-	In step 2, instead of clause 6.6.2.3A.2.4.3  use clause 6.6.2.3A.2_2.4.3.

-	In step 4, instead of Table 6.6.2.3A.2.4.1-1  use Table 6.6.2.3A.2_2.4.1-1.

-	In step 6, instead of clause 6.2.3A.2.5-1  use Table 6.2.3A.2_2.5-1.

6.6.2.3A.2_2.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

6.6.2.3A.2_2.5	Test Requirements

Same test requirements as in clause 6.6.2.3A.2.5 with following exception:

-	Instead of Table 6.2.3A.2.5-1  use Table 6.2.3A.2_2.5-1.

##### 6.6.2.3A.3	Adjacent Channel Leakage power Ratio for CA (intra-band non-contiguous DL CA and UL CA)

###### 6.6.2.3A.3.1	Test purpose

Same test purpose as defined in clause 6.6.2.3A.1.1.

6.6.2.3A.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 11 and forward that support intra-band non-contiguous DL CA and UL CA but do not support UL 64QAM or UL 256QAM..

6.6.2.3A.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.2.3A.0.

6.6.2.3A.3.4	Test description

6.6.2.3A.3.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.3A.3.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.3A.3.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Refer to test points A: Low Wgap, Maximum Wgap |  |  |  |  |
| Test CC Combination setting (NRB) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  | Refer to test point Test only test points with Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  | Wgap [MHz] | DL Allocation |  | UL Allocation |  |  |  |
|  | PCC NRB | SCCs NRB |  | CC MOD | PCC & SCC RB allocation | CC MOD | PCC NRB_alloc (LCRB @ RBstart) | CC MOD | SCC NRB_alloc (LCRB @ RBstart) |
| Test Parameters for CA_4A-4A Configurations |  |  |  |  |  |  |  |  |  |
| 1 | 25 | 25 | 35 | N/A | N/A | 16QAM | P_1@0 | 16QAM | S_1@24 |
| 2 | 25 | 25 | 35 | N/A | N/A | 16QAM | P_25@0 | 16QAM | S_25@0 |
| 3 | 25 | 25 | 5 | N/A | N/A | 16QAM | P_1@0 | 16QAM | S_1@24 |
| 4 | 25 | 25 | 5 | N/A | N/A | 16QAM | P_25@0 | 16QAM | S_25@0 |
| 5 | 100 | 100 | 5 | N/A | N/A | 16QAM | P_1@0 | 16QAM | S_1@99 |
| 6 | 100 | 100 | 5 | N/A | N/A | 16QAM | P_25@0 | 16QAM | S_25@75 |
| 7 | 100 | 100 | 5 | N/A | N/A | 16QAM | P_100@0 | 16QAM | S_100@0 |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.3A.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.3A.1.4.3.

6.6.2.3A.3.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.6.2.3A.1.4.3

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.6.2.3A.3.4.1-1 on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control “up” commands in every uplink scheduling information to the UE until the UE transmits at PUMAX level.

6.	Measure the mean power over all component carriers of the UE in the CA configuration of the radio access mode according to the test configuration, which shall meet the requirements described in Table 6.2.3A.3.5-1. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

7.	Measure the rectangular filtered mean power for E-UTRA using a rms detector.

8.	Measure the rectangular filtered mean power of the CA E-UTRA adjacent channel on lower side of the lowest sub-clock, and the higher side of the highest sub-block (out of gap measurement) using a rms detector.

9.	If Wgap is larger or equal to either of the sub-block bandwidths, perform this step, otherwise continue to next step. Measure the rectangular filtered mean power of the CA E-UTRA adjacent channel on higher side of the lowest sub-clock, and the lower side of the highest sub-block (in gap measurement).

10.	Measure the RRC filtered mean power of the first and the second UTRA adjacent channel on lower side of the lowest sub-clock, and the higher side of the highest sub-block (out of gap measurement) using a rms detector.

11.	If Wgap is larger or equal to 5 MHz, perform this step, otherwise continue to next step. Measure the rectangular filtered mean power of the first UTRA adjacent channel on higher side of the lowest sub-clock, and the lower side of the highest sub-block (in gap measurement).

12.	If Wgap is larger or equal to 15 MHz, perform this step, otherwise continue to next step. Measure the rectangular filtered mean power of the second UTRA adjacent channel on higher side of the lowest sub-clock, and the lower side of the highest sub-block (in gap measurement).

13.	Calculate the ratio of the power between the values measured in step 7 over step 8 for CA E-UTRAACLR (out of gap measurement).

14.	Calculate the ratio of the power between the values measured in step 7 over step 9 for CA E-UTRAACLR (in gap measurement).

15.	Calculate the ratio of the power between the values measured in step 7 over step 10 for UTRAACLR1, UTRAACLR2 (out of gap measurement).

16.	Calculate the ratio of the power between the values measured in step 7 over step 11 and 12 for UTRAACLR1, UTRAACLR2 (in gap measurement).

6.6.2.3A.3.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

6.6.2.3A.3.5	Test Requirements

6.6.2.3A.3.5.1	Test requirements E-UTRA

-	The measured UE mean power, derived in step 6, shall fulfil requirements in Table 6.2.3A.3.5-1 as appropriate,

and

-	if the measured adjacent channel power is greater than –50 dBm then the measured E-UTRAACLR, derived in step 13 and 14, shall be higher than the limits in table 6.6.2.3A.3.5.1-1.

Table 6.6.2.3A.3.5.1-1: E-UTRA UE ACLR

|  | Channel bandwidth / E-UTRAACLR1 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UE channel | +1.4 MHz or -1.4 MHz | +3 MHz or -3 MHz | +5MHz or -5MHz | +10MHz or -10MHz | +15MHz or -15MHz | +20MHz or -20MHz |

6.6.2.3A.3.5.2 Test requirements UTRA

If the measured UTRA channel power is greater than –50dBm then the measured UTRAACLR1, UTRAACLR2, derived in steps 15 and 16, shall be higher than the limits in table 6.6.2.3A.3.5.2-1.

Table 6.6.2.3A.3.5.2-1: UTRA UE ACLR

|  | Channel bandwidth / UTRAACLR1/2 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| UTRAACLR1 | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB |
| Adjacent channel centre frequency offset (in MHz) | 0.7+BWUTRA/2 / -0.7-BWUTRA/2 | 1.5+BWUTRA/2 / -1.5-BWUTRA/2 | 2.5+BWUTRA/2 / -2.5-BWUTRA/2 | 5+BWUTRA/2 / -5-BWUTRA/2 | 7.5+BWUTRA/2 / -7.5-BWUTRA/2 | 10+BWUTRA/2 / -10-BWUTRA/2 |
| UTRAACLR2 | - | - | 35.2 dB | 35.2 dB | 35.2 dB | 35.2 dB |
| Adjacent channel centre frequency offset (in MHz) | - | - | 2.5+3*BWUTRA/2 / -2.5-3*BWUTRA/2 | 5+3*BWUTRA/2 / -5-3*BWUTRA/2 | 7.5+3*BWUTRA/2 / -7.5-3*BWUTRA/2 | 10+3*BWUTRA/2 / -10-3*BWUTRA/2 |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UTRA 5MHz channel Measurement bandwidth1 | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz |
| UTRA 1.6MHz channel measurement bandwidth2 | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz |
| Note 1: Applicable for E-UTRA FDD co-existence with UTRA FDD in paired spectrum. Note 2: Applicable for E-UTRA TDD co-existence with UTRA TDD in unpaired spectrum. Note 3: BWUTRA for UTRA FDD is 5MHz and for UTRA TDD is 1.6MHz. |  |  |  |  |  |  |

##### 6.6.2.3A.3_1	Adjacent Channel Leakage power Ratio for CA (intra-band non-contiguous DL CA and UL CA) for UL 64QAM

6.6.2.3A.3_1.1	Test purpose

Same test purpose as defined in clause 6.6.2.3A.1.1.

6.6.2.3A.3_1.2	Test applicability

This test case applies to all types of E-UTRA UE release 13 and forward that support intra-band non-contiguous DL CA and UL CA and UL 64QAM.

6.6.2.3A.3_1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.2.3A.0.

6.6.2.3A.3_1.4	Test description

6.6.2.3A.3_1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.3A.3_1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.3A.3_1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Refer to test points A: Mid frequency, Maximum Wgap(low + high frequency) |  |  |  |  |
| Test CC Combination setting (NRB) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  | Refer to test point Test only test points with Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  | Wgap [MHz] | DL Allocation |  | UL Allocation |  |  |  |
|  | PCC NRB | SCCs NRB |  | CC MOD | PCC & SCC RB allocation | CC MOD | PCC NRB_alloc (LCRB @ RBstart) | CC MOD | SCC NRB_alloc (LCRB @ RBstart) |
| Test Parameters for CA_4A-4A Configurations |  |  |  |  |  |  |  |  |  |
| 1 | 25 | 25 | 35 | N/A | N/A | 64QAM | P_1@0 | 64QAM | S_1@24 |
| 2 | 25 | 25 | 35 | N/A | N/A | 64QAM | P_25@0 | 64QAM | S_25@0 |
| 3 | 25 | 25 | 5 | N/A | N/A | 64QAM | P_1@0 | 64QAM | S_1@24 |
| 4 | 25 | 25 | 5 | N/A | N/A | 64QAM | P_25@0 | 64QAM | S_25@0 |
| 5 | 100 | 100 | 5 | N/A | N/A | 64QAM | P_1@0 | 64QAM | S_1@99 |
| 6 | 100 | 100 | 5 | N/A | N/A | 64QAM | P_25@0 | 64QAM | S_25@75 |
| 7 | 100 | 100 | 5 | N/A | N/A | 64QAM | P_100@0 | 64QAM | S_100@0 |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.3A.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.3A.1.4.3.

6.6.2.3A.3_1.4.2	Test procedure

Same test procedure as in clause 6.6.2.3A.3.4.2.

6.6.2.3A.3_1.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

6.6.2.3A.3_1.5	Test Requirements

Same test requirements as in clause 6.6.2.3A.3.5.

##### 6.6.2.3A.3_2	Adjacent Channel Leakage power Ratio for CA (intra-band non-contiguous DL CA and UL CA) for UL 256QAM

6.6.2.3A.3_2.1	Test purpose

Same as in clause 6.6.2.3A.1.1.

6.6.2.3A.3_2.2	Test applicability

This test case applies to all types of E-UTRA UE release 14 and forward that support intra-band non-contiguous DL CA and UL CA and UL 256QAM.

6.6.2.3A.3_2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.2.3A.0.

6.6.2.3A.3_2.4	Test description

6.6.2.3A.3_2.4.1	Initial condition

Same as in clause 6.6.2.3A.3_1.4.1 with the following exception:

-	In Step 4, instead of Table 6.6.2.3A.3_1.4.1-1  use Table 6.6.2.3A.3_2.4.1-1

Table 6.6.2.3A.3_2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Refer to test points A: Mid frequency, Maximum Wgap(low + high frequency) |  |  |  |  |
| Test CC Combination setting (NRB) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  | Refer to test point Test only test points with Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  | Wgap [MHz] | DL Allocation |  | UL Allocation |  |  |  |
|  | PCC NRB | SCCs NRB |  | CC MOD | PCC & SCC RB allocation | CC MOD | PCC NRB_alloc (LCRB @ RBstart) | CC MOD | SCC NRB_alloc (LCRB @ RBstart) |
| Test Parameters for CA_4A-4A Configurations |  |  |  |  |  |  |  |  |  |
| 1 | 25 | 25 | 35 | N/A | N/A | 256QAM | P_1@0 | 256QAM | S_1@24 |
| 2 | 25 | 25 | 35 | N/A | N/A | 256QAM | P_25@0 | 256QAM | S_25@0 |
| 3 | 25 | 25 | 5 | N/A | N/A | 256QAM | P_1@0 | 256QAM | S_1@24 |
| 4 | 25 | 25 | 5 | N/A | N/A | 256QAM | P_25@0 | 256QAM | S_25@0 |
| 5 | 100 | 100 | 5 | N/A | N/A | 256QAM | P_1@0 | 256QAM | S_1@99 |
| 6 | 100 | 100 | 5 | N/A | N/A | 256QAM | P_25@0 | 256QAM | S_25@75 |
| 7 | 100 | 100 | 5 | N/A | N/A | 256QAM | P_100@0 | 256QAM | S_100@0 |

6.6.2.3A.3_2.4.2	Test procedure

Same as in clause 6.6.2.3A.3_1.4.2.

6.6.2.3A.3_2.4.3	Message contents

Same as in clause 6.6.2.3A.3_1.4.3.

6.6.2.3A.3_2.5	Test Requirements

Same as in clause 6.6.2.3A.3_1.5.

##### 6.6.2.3A.4	Adjacent Channel Leakage power Ratio for CA (3UL CA)

6.6.2.3A.4.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR) for CA.

6.6.2.3A.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 13 and forward that support 3DL CA and 3UL CA.

6.6.2.3A.4.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.2.3A.0.

6.6.2.3A.4.4	Test description

6.6.2.3A.4.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on E-UTRA CA configurations specified in table 5.4.2A.1-1 and 5.4.2A.1-2. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 6.6.2.3A.4.4.1-1 for intra-band contiguous CA, 6.6.2.3A.4.4.1-2 for inter-band CA and 6.6.2.3A.4.4.1-3 for intra-band non-contiguous CA. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.3A.4.4.1-1: Test Configuration Table for intra-band contiguous CA

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Low range, High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  | Lowest NRB_agg, Highest NRB_agg (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCC1 NRB | SCC2 NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations(LCRB @ RBstart) |  |  |  |
| 50 | 100 | 100 | N/A for this test | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | - |
| 50 | 100 | 100 |  | QPSK | 200 | P_50@0 | P_100@0 | P_100@0 | - |
| 50 | 100 | 100 |  | 16QAM | 8 | P_8@0 | S_0@0 | S_0@0 | - |
| 50 | 100 | 100 |  | 16QAM | 200 | P_50@0 | P_100@0 | P_100@0 | - |
| 75 | 100 | 100 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | - |
| 75 | 100 | 100 |  | QPSK | 275 | P_75@0 | P_100@0 | P_100@0 | - |
| 75 | 100 | 100 |  | 16QAM | 8 | P_8@0 | S_0@0 | S_0@0 | - |
| 75 | 100 | 100 |  | 16QAM | 275 | P_75@0 | P_100@0 | P_100@0 | - |
| 100 | 100 | 100 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | - |
| 100 | 100 | 100 |  | QPSK | 300 | P_100@0 | P_100@0 | P_100@0 | - |
| 100 | 100 | 100 |  | 16QAM | 8 | P_8@0 | S_0@0 | S_0@0 | - |
| 100 | 100 | 100 |  | 16QAM | 300 | P_100@0 | P_100@0 | P_100@0 | - |
| 100 | 50 | 100 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | - |
| 100 | 50 | 100 |  | QPSK | 250 | P_100@0 | P_50@0 | P_100@0 | - |
| 100 | 50 | 100 |  | 16QAM | 8 | P_8@0 | S_0@0 | S_0@0 | - |
| 100 | 50 | 100 |  | 16QAM | 250 | P_100@0 | P_50@0 | P_100@0 | - |
| 100 | 75 | 100 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | - |
| 100 | 75 | 100 |  | QPSK | 275 | P_100@0 | P_75@0 | P_100@0 | - |
| 100 | 75 | 100 |  | 16QAM | 8 | P_8@0 | S_0@0 | S_0@0 | - |
| 100 | 75 | 100 |  | 16QAM | 275 | P_100@0 | P_75@0 | P_100@0 | - |
| 100 | 100 | 50 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | - |
| 100 | 100 | 50 |  | QPSK | 250 | P_100@0 | P_100@0 | P_50@0 | - |
| 100 | 100 | 50 |  | 16QAM | 8 | P_8@0 | S_0@0 | S_0@0 | - |
| 100 | 100 | 50 |  | 16QAM | 250 | P_100@0 | P_100@0 | P_50@0 | - |
| 100 | 100 | 75 |  | QPSK | 8 | P_8@0 | S_0@0 | S_0@0 | - |
| 100 | 100 | 75 |  | QPSK | 275 | P_100@0 | P_100@0 | P_75@0 | - |
| 100 | 100 | 75 |  | 16QAM | 8 | P_8@0 | S_0@0 | S_0@0 | - |
| 100 | 100 | 75 |  | 16QAM | 275 | P_100@0 | P_100@0 | P_75@0 | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg , only the first of those is tested, according to the order on the Test Configuration Table list. |  |  |  |  |  |  |  |  |  |

Table 6.6.2.3A.4.4.1-2: Test Configuration Table for inter-band CA

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes |  |  |  |  | Low and High range for both PCC and SCC (Note 3) |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  |  | DL Allocation | CC MOD | UL Allocation |  |  |  |
| PCC | SCC1 | SCC2 | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCCs RB allocations |  |  |
| NRB | NRB | NRB |  |  |  | PCC | SCCs |  |
| 25 | 100 | 100 | N/A for this test | QPSK | 44 | P_8@0 | S_18@0 | S_18@0 |
| 25 | 100 | 100 |  | QPSK | 225 | P_25@0 | S_100@0 | S_100@0 |
| 25 | 100 | 100 |  | 16QAM | 44 | P_8@0 | S_18@0 | S_18@0 |
| 25 | 100 | 100 |  | 16QAM | 225 | P_25@0 | S_100@0 | S_100@0 |
| 50 | 100 | 100 |  | QPSK | 48 | P_12@0 | S_18@0 | S_18@0 |
| 50 | 100 | 100 |  | QPSK | 250 | P_50@0 | S_100@0 | S_100@0 |
| 50 | 100 | 100 |  | 16QAM | 48 | P_12@0 | S_18@0 | S_18@0 |
| 50 | 100 | 100 |  | 16QAM | 250 | P_50@0 | S_100@0 | S_100@0 |
| 75 | 100 | 100 |  | QPSK | 52 | P_16@0 | S_18@0 | S_18@0 |
| 75 | 100 | 100 |  | QPSK | 275 | P_75@0 | S_100@0 | S_100@0 |
| 75 | 100 | 100 |  | 16QAM | 52 | P_16@0 | S_18@0 | S_18@0 |
| 75 | 100 | 100 |  | 16QAM | 275 | P_75@0 | S_100@0 | S_100@0 |
| 100 | 100 | 100 |  | QPSK | 54 | P_18@0 | S_18@0 | S_18@0 |
| 100 | 100 | 100 |  | QPSK | 300 | P_100@0 | S_100@0 | S_100@0 |
| 100 | 100 | 100 |  | 16QAM | 54 | P_18@0 | S_18@0 | S_18@0 |
| 100 | 100 | 100 |  | 16QAM | 300 | P_100@0 | S_100@0 | S_100@0 |
| 100 | 25 | 100 |  | QPSK | 44 | P_18@0 | S_8@0 | S_18@0 |
| 100 | 25 | 100 |  | QPSK | 225 | P_100@0 | S_25@0 | S_100@0 |
| 100 | 25 | 100 |  | 16QAM | 44 | P_18@0 | S_8@0 | S_18@0 |
| 100 | 25 | 100 |  | 16QAM | 225 | P_100@0 | S_25@0 | S_100@0 |
| 100 | 50 | 100 |  | QPSK | 48 | P_18@0 | S_12@0 | S_18@0 |
| 100 | 50 | 100 |  | QPSK | 250 | P_100@0 | S_50@0 | S_100@0 |
| 100 | 50 | 100 |  | 16QAM | 48 | P_18@0 | S_12@0 | S_18@0 |
| 100 | 50 | 100 |  | 16QAM | 250 | P_100@0 | S_50@0 | S_100@0 |
| 100 | 75 | 100 |  | QPSK | 52 | P_18@0 | S_16@0 | S_18@0 |
| 100 | 75 | 100 |  | QPSK | 275 | P_100@0 | S_75@0 | S_100@0 |
| 100 | 75 | 100 |  | 16QAM | 52 | P_18@0 | S_16@0 | S_18@0 |
| 100 | 75 | 100 |  | 16QAM | 275 | P_100@0 | S_75@0 | S_100@0 |
| 50 | 50 | 100 |  | QPSK | 42 | P_12@0 | S_12@0 | S_18@0 |
| 50 | 50 | 100 |  | QPSK | 200 | P_50@0 | S_50@0 | S_100@0 |
| 50 | 50 | 100 |  | 16QAM | 42 | P_12@0 | S_12@0 | S_18@0 |
| 50 | 50 | 100 |  | 16QAM | 200 | P_50@0 | S_50@0 | S_100@0 |
| 50 | 75 | 100 |  | QPSK | 46 | P_12@0 | S_16@0 | S_18@0 |
| 50 | 75 | 100 |  | QPSK | 225 | P_50@0 | S_75@0 | S_100@0 |
| 50 | 75 | 100 |  | 16QAM | 46 | P_12@0 | S_16@0 | S_18@0 |
| 50 | 75 | 100 |  | 16QAM | 225 | P_50@0 | S_75@0 | S_100@0 |
| 75 | 50 | 100 |  | QPSK | 46 | P_16@0 | S_12@0 | S_18@0 |
| 75 | 50 | 100 |  | QPSK | 225 | P_50@0 | S_75@0 | S_100@0 |
| 75 | 50 | 100 |  | 16QAM | 46 | P_16@0 | S_12@0 | S_18@0 |
| 75 | 50 | 100 |  | 16QAM | 225 | P_50@0 | S_75@0 | S_100@0 |
| 75 | 75 | 100 |  | QPSK | 50 | P_16@0 | S_16@0 | S_18@0 |
| 75 | 75 | 100 |  | QPSK | 250 | P_75@0 | S_75@0 | S_100@0 |
| 75 | 75 | 100 |  | 16QAM | 50 | P_16@0 | S_16@0 | S_18@0 |
| 75 | 75 | 100 |  | 16QAM | 250 | P_75@0 | S_75@0 | S_100@0 |
| 50 | 100 | 50 |  | QPSK | 42 | P_12@0 | S_18@0 | S_12@0 |
| 50 | 100 | 50 |  | QPSK | 200 | P_50@0 | S_100@0 | S_50@0 |
| 50 | 100 | 50 |  | 16QAM | 42 | P_12@0 | S_18@0 | S_12@0 |
| 50 | 100 | 50 |  | 16QAM | 200 | P_50@0 | S_100@0 | S_50@0 |
| 50 | 100 | 75 |  | QPSK | 46 | P_12@0 | S_18@0 | S_16@0 |
| 50 | 100 | 75 |  | QPSK | 225 | P_50@0 | S_100@0 | S_75@0 |
| 50 | 100 | 75 |  | 16QAM | 46 | P_12@0 | S_18@0 | S_16@0 |
| 50 | 100 | 75 |  | 16QAM | 225 | P_50@0 | S_100@0 | S_75@0 |
| 75 | 100 | 50 |  | QPSK | 46 | P_16@0 | S_18@0 | S_12@0 |
| 75 | 100 | 50 |  | QPSK | 225 | P_75@0 | S_100@0 | S_50@0 |
| 75 | 100 | 50 |  | 16QAM | 46 | P_16@0 | S_18@0 | S_12@0 |
| 75 | 100 | 50 |  | 16QAM | 225 | P_75@0 | S_100@0 | S_50@0 |
| 75 | 100 | 75 |  | QPSK | 50 | P_16@0 | S_18@0 | S_16@0 |
| 75 | 100 | 75 |  | QPSK | 250 | P_75@0 | S_100@0 | S_75@0 |
| 75 | 100 | 75 |  | 16QAM | 50 | P_16@0 | S_18@0 | S_16@0 |
| 75 | 100 | 75 |  | 16QAM | 250 | P_75@0 | S_100@0 | S_75@0 |
| 75 | 75 | 50 |  | QPSK | 44 | P_16@0 | S_16@0 | S_12@0 |
| 75 | 75 | 50 |  | QPSK | 200 | P_75@0 | S_75@0 | S_50@0 |
| 75 | 75 | 50 |  | 16QAM | 44 | P_16@0 | S_16@0 | S_12@0 |
| 75 | 75 | 50 |  | 16QAM | 200 | P_75@0 | S_75@0 | S_50@0 |
| 75 | 75 | 75 |  | QPSK | 48 | P_16@0 | S_16@0 | S_16@0 |
| 75 | 75 | 75 |  | QPSK | 225 | P_75@0 | S_75@0 | S_75@0 |
| 75 | 75 | 75 |  | 16QAM | 48 | P_16@0 | S_16@0 | S_16@0 |
| 75 | 75 | 75 |  | 16QAM | 225 | P_75@0 | S_75@0 | S_75@0 |
| 100 | 50 | 50 |  | QPSK | 42 | P_18@0 | S_12@0 | S_12@0 |
| 100 | 50 | 50 |  | QPSK | 200 | P_100@0 | S_50@0 | S_50@0 |
| 100 | 50 | 50 |  | 16QAM | 42 | P_18@0 | S_12@0 | S_12@0 |
| 100 | 50 | 50 |  | 16QAM | 200 | P_100@0 | S_50@0 | S_50@0 |
| 100 | 50 | 75 |  | QPSK | 46 | P_18@0 | S_12@0 | S_16@0 |
| 100 | 50 | 75 |  | QPSK | 225 | P_100@0 | S_50@0 | S_75@0 |
| 100 | 50 | 75 |  | 16QAM | 46 | P_18@0 | S_12@0 | S_16@0 |
| 100 | 50 | 75 |  | 16QAM | 225 | P_100@0 | S_50@0 | S_75@0 |
| 100 | 75 | 50 |  | QPSK | 46 | P_18@0 | S_16@0 | S_12@0 |
| 100 | 75 | 50 |  | QPSK | 225 | P_100@0 | S_75@0 | S_50@0 |
| 100 | 75 | 50 |  | 16QAM | 46 | P_18@0 | S_16@0 | S_12@0 |
| 100 | 75 | 50 |  | 16QAM | 225 | P_100@0 | S_75@0 | S_50@0 |
| 100 | 75 | 75 |  | QPSK | 50 | P_18@0 | S_16@0 | S_16@0 |
| 100 | 75 | 75 |  | QPSK | 250 | P_100@0 | S_75@0 | S_75@0 |
| 100 | 75 | 75 |  | 16QAM | 50 | P_18@0 | S_16@0 | S_16@0 |
| 100 | 75 | 75 |  | 16QAM | 250 | P_100@0 | S_75@0 | S_75@0 |
| 100 | 100 | 50 |  | QPSK | 48 | P_18@0 | S_18@0 | S_12@0 |
| 100 | 100 | 50 |  | QPSK | 250 | P_100@0 | S_100@0 | S_50@0 |
| 100 | 100 | 50 |  | 16QAM | 48 | P_18@0 | S_18@0 | S_12@0 |
| 100 | 100 | 50 |  | 16QAM | 250 | P_100@0 | S_100@0 | S_50@0 |
| 100 | 100 | 75 |  | QPSK | 52 | P_18@0 | S_18@0 | S_16@0 |
| 100 | 100 | 75 |  | QPSK | 275 | P_100@0 | S_100@0 | S_75@0 |
| 100 | 100 | 75 |  | 16QAM | 52 | P_18@0 | S_18@0 | S_16@0 |
| 100 | 100 | 75 |  | 16QAM | 275 | P_100@0 | S_100@0 | S_75@0 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg. only the first of those is tested. according to the order on the Test Configuration Table list. Note 3: For Low range place test point with RBstart=0. For High range place test point with RBstart=RBmax-LCRB+1. |  |  |  |  |  |  |  |  |

Table 6.6.2.3A.4.4.1-3: Test Configuration Table for intra-band non-contiguous CA

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Refer to test points A: Low Wgap, Maximum Wgap |  |  |  |  |  |
| Test CC Combination setting (NRB) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  | Refer to test point Test only test points with Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  | Wgap [MHz] | DL Allocation |  | UL Allocation |  |  |  |  |
|  | PCC NRB | SCCs NRB |  | CC MOD | PCC & SCC RB allocation | CC MOD | PCC NRB_alloc (LCRB @ RBstart) | CC MOD | SCC NRB_alloc (LCRB @ RBstart) |  |
| Test Parameters for CA_4A-4A Configurations |  |  |  |  |  |  |  |  |  |  |
| 1 | 25 | 25 | 35 | N/A | N/A | 16QAM | P_1@0 | 16QAM | S_1@24 | S_1@24 |
| 2 | 25 | 25 | 35 | N/A | N/A | 16QAM | P_25@0 | 16QAM | S_25@0 | S_25@0 |
| 3 | 25 | 25 | 5 | N/A | N/A | 16QAM | P_1@0 | 16QAM | S_1@24 | S_1@24 |
| 4 | 25 | 25 | 5 | N/A | N/A | 16QAM | P_25@0 | 16QAM | S_25@0 | S_25@0 |
| 5 | 100 | 100 | 5 | N/A | N/A | 16QAM | P_1@0 | 16QAM | S_1@99 | S_1@99 |
| 6 | 100 | 100 | 5 | N/A | N/A | 16QAM | P_25@0 | 16QAM | S_25@75 | S_25@75 |
| 7 | 100 | 100 | 5 | N/A | N/A | 16QAM | P_100@0 | 16QAM | S_100@0 | S_100@0 |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.6.2.3A.4.4.1-1 for intra-band contiguous CA, 6.6.2.3A.4.4.1-2 for inter-band CA and 6.6.2.3A.4.4.1-3 for intra-band non-contiguous CA.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.3A.4.4.3.

6.6.2.3A.4.4.2	Test procedure

1.	Configure SCCs according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCCs as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.6.2.3A.1.4.3

3.	SS activates SCCs by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.6.2.3A.1.4.1-1 for intra-band contiguous CA, 6.6.2.3A.4.4.1-2 for inter-band CA and 6.6.2.3A.4.4.1-3 for intra-band non-contiguous CA on all PCC and SCCs. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control “up” commands in every uplink scheduling information to the UE until the UE transmits at PUMAX level.

6.	Measure the mean power over all component carriers of the UE in the CA configuration of the radio access mode according to the test configuration, which shall meet the requirements described in Table 6.2.3A.1.5-1 for intra-band non-contiguous CA, 6.2.3A.2.5-1 for inter-band CA and 6.2.3A.3.5-1 for intra-band non-contiguous CA. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

7.	Measure the rectangular filtered mean power for CA E-UTRA (in the first band for inter-band CA UE) using a rms detector.

8.	For intra-band non-contiguous 3UL CA UE, execute steps 8a) to 8d):

8a)	Measure the rectangular filtered mean power of the first CA E-UTRA adjacent channel on both lower and upper side of the CA E-UTRA channel using a rms detector, respectively.

8b)	Measure the RRC filtered mean power of the first and the second UTRA adjacent channel on both lower and upper side of the CA E-UTRA channel using a rms detector, respectively.

8c)	Calculate the ratio of the power between the values measured in step 7 over step 8a) for CA E-UTRAACLR.

8d)	Calculate the ratio of the power between the values measured in step 7 over step 8b) for UTRAACLR1, UTRAACLR2

9.	For inter-band 3UL CA UE, execute steps 9a) to 9e):

9a)	Measure the rectangular filtered mean power of the first CA E-UTRA adjacent channel on both lower and upper side of the CA E-UTRA channel in the first band of the CA configuration using a rms detector, respectively

9b)	Measure the RRC filtered mean power of the first and the second UTRA adjacent channel on both lower and upper side of the CA E-UTRA channel in the first band of the CA configuration using a rms detector, respectively.

9c)	Calculate the ratio of the power between the values measured in step 7 over step 9a) for CA E-UTRAACLR.

9d)	Calculate the ratio of the power between the values measured in step 7 over step 9b) for UTRAACLR1, UTRAACLR2.

9e)	Repeat steps 6 to 11 for the second band of the CA configuration.

10.	For intra-band non-contiguous 3UL CA UE, execute steps 10a) to 10i):

10a)	Measure the rectangular filtered mean power of the CA E-UTRA adjacent channel on lower side of the lowest sub-clock, and the higher side of the highest sub-block (out of gap measurement) using a rms detector.

10b)	If Wgap is larger or equal to either of the sub-block bandwidths, perform this step, otherwise continue to next step. Measure the rectangular filtered mean power of the CA E-UTRA adjacent channel on higher side of the lowest sub-clock, and the lower side of the highest sub-block (in gap measurement).

10c)	Measure the RRC filtered mean power of the first and the second UTRA adjacent channel on lower side of the lowest sub-clock, and the higher side of the highest sub-block (out of gap measurement) using a rms detector.

10d)	If Wgap is larger or equal to 5 MHz, perform this step, otherwise continue to next step. Measure the rectangular filtered mean power of the first UTRA adjacent channel on higher side of the lowest sub-clock, and the lower side of the highest sub-block (in gap measurement).

10e)	If Wgap is larger or equal to 15 MHz, perform this step, otherwise continue to next step. Measure the rectangular filtered mean power of the second UTRA adjacent channel on higher side of the lowest sub-clock, and the lower side of the highest sub-block (in gap measurement).

10f)	Calculate the ratio of the power between the values measured in step 7 over step 10a) for CA E-UTRAACLR (out of gap measurement).

10g)	Calculate the ratio of the power between the values measured in step 7 over step 10b) for CA E-UTRAACLR (in gap measurement).

10h)	Calculate the ratio of the power between the values measured in step 7 over step 10c) for UTRAACLR1, UTRAACLR2 (out of gap measurement).

10i)	Calculate the ratio of the power between the values measured in step 7 over step 10d) and 10e) for UTRAACLR1, UTRAACLR2 (in gap measurement).

6.6.2.3A.4.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

6.6.2.3A.4.5	Test Requirements

6.6.2.3A.4.5.1	Test requirements of UTRA for CA

If the measured UTRA channel power is greater than –50dBm then the measured UTRAACLR1 and UTRAACLR2, derived in steps 8d), 9d) 10h)and 10i), shall be higher than the limits in table 6.6.2.3A.4.5.1-1.

Table 6.6.2.3A.4.5.1-1: UTRA UE ACLR for CA

|  | CA bandwidth class / UTRAACLR1/2 / measurement bandwidth |
| --- | --- |
|  | CA bandwidth class D |
| UTRAACLR1 | 32.2 dB |
| Adjacent channel centre frequency offset (in MHz) | + BWChannel_CA /2 + BWUTRA/2 / - BWChannel_CA / 2 - BWUTRA/2 |
| UTRAACLR2 | 35.2 dB |
| Adjacent channel centre frequency offset (in MHz) | + BWChannel_CA /2 + 3*BWUTRA/2 / - BWChannel_CA /2 – 3*BWUTRA/2 |
| CA E-UTRA channel Measurement bandwidth | BWChannel_CA - 2* BWGB |
| UTRA 5MHz channel Measurement bandwidth (Note 1) | 3.84 MHz |
| UTRA 1.6MHz channel measurement bandwidth (Note 2) | 1.28 MHz |
| Note 1: Applicable for E-UTRA FDD co-existence with UTRA FDD in paired spectrum. Note 2: Applicable for E-UTRA TDD co-existence with UTRA TDD in unpaired spectrum. |  |

6.6.2.3A.4.5.2	Test requirements of CA E-UTRA

-	The measured UE mean power in the channel bandwidth as specified in clause 5.4.2A, derived in step 6, shall fulfil requirements in Table 6.2.3A.1.5-1 for intra-band non-contiguous CA, 6.2.3A.2.5-1 for inter-band CA and 6.2.3A.3.5-1 for intra-band non-contiguous CA as appropriate,

and

-	if the measured adjacent channel power is greater than –50 dBm then the measured CA E-UTRAACLR, derived in steps 8c), 9c) 10f)and 10g), shall be higher than the limits in table 6.6.2.3A.4.5.2-1.

Table 6.6.2.3A.4.5.2-1: CA E-UTRA ACLR

|  | CA bandwidth class / CA E-UTRAACLR / Measurement bandwidth |
| --- | --- |
|  | CA bandwidth class D |
| CA E-UTRAACLR | 29.2 dB |
| CA E-UTRA channel Measurement bandwidth | BWChannel_CA - 2* BWGB |
| Adjacent channel centre frequency offset (in MHz) | + BWChannel_CA / - BWChannel_CA |

##### 6.6.2.3A.5	Adjacent Channel Leakage power Ratio for CA (4UL CA)

6.6.2.3A.5.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR) for CA.

6.6.2.3A.5.2	Test applicability

This test case applies to all types of E-UTRA UE release 11 and forward that support inter-band and 4UL CA.

6.6.2.3A.5.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.2.3A.0.

6.6.2.3A.5.4	Test description

6.6.2.3A.5.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA CA configurations specified in table 5.4.2A.1-2. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and are shown in table 6.6.2.3A.5.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.3A.5.4.1-1: inter-band CA Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  |  |  | Low and High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE (Note 1). |  |  |  |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg DL Allocation |  |  |  |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| ID | PCC NRB | SCC1 NRB | SCC2 NRB | SCC3 NRB | PCC & SCC RB allocation |  | NRB_ alloc | PCC & SCC1 & SCC2 & SCC3 RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 100 | 50 | 100 | 50 |  | QPSK | 24 | P_12@0 | S_0@0 | S_12@0 | S_0@0 |
| 2 | 100 | 50 | 100 | 50 |  | QPSK | 100 | P_50@0 | S_0@0 | S_50@0 | S_0@0 |
| 3 | 100 | 50 | 100 | 50 |  | QPSK | 300 | P_100@0 | S_50@0 | S_100@0 | S_50@0 |
| 4 | 100 | 50 | 100 | 50 |  | 16 QAM | 24 | P_12@0 | S_0@0 | S_12@0 | S_0@0 |
| 5 | 100 | 50 | 100 | 50 |  | 16 QAM | 100 | P_50@0 | S_0@0 | S_50@0 | S_0@0 |
| 6 | 100 | 50 | 100 | 50 |  | 16 QAM | 300 | P_100@0 | S_50@0 | S_100@0 | S_50@0 |
| 7 | 100 | 50 | 100 | 50 |  | 64 QAM | 24 | P_12@0 | S_0@0 | S_12@0 | S_0@0 |
| 8 | 100 | 50 | 100 | 50 |  | 64 QAM | 300 | P_100@0 | S_50@0 | S_100@0 | S_50@0 |
| 9 | 100 | 50 | 100 | 50 | N/A for this | 256 QAM | 300 | P_100@0 | S_50@0 | S_100@0 | S_50@0 |
| 10 | 100 | 100 | 100 | 100 | test | QPSK | 24 | P_12@0 | S_0@0 | S_12@0 | S_0@0 |
| 11 | 100 | 100 | 100 | 100 |  | QPSK | 100 | P_50@0 | S_0@0 | S_50@0 | S_0@0 |
| 12 | 100 | 100 | 100 | 100 |  | QPSK | 400 | P_100@0 | S_100@0 | S_100@0 | S_100@0 |
| 13 | 100 | 100 | 100 | 100 |  | 16 QAM | 24 | P_12@0 | S_0@0 | S_12@0 | S_0@0 |
| 14 | 100 | 100 | 100 | 100 |  | 16 QAM | 100 | P_50@0 | S_0@0 | S_50@0 | S_0@0 |
| 15 | 100 | 100 | 100 | 100 |  | 16 QAM | 400 | P_100@0 | S_100@0 | S_100@0 | S_100@0 |
| 16 | 100 | 100 | 100 | 100 |  | 64 QAM | 24 | P_12@0 | S_0@0 | S_12@0 | S_0@0 |
| 17 | 100 | 100 | 100 | 100 |  | 64 QAM | 300 | P_100@0 | S_50@0 | S_100@0 | S_50@0 |
| 18 | 100 | 100 | 100 | 100 |  | 256 QAM | 400 | P_100@0 | S_100@0 | S_100@0 | S_100@0 |
| NOTE 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. NOTE 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure group A.55 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] clause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C0, C.1 and C.3.0 and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.1A.5.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.3A.5.4.3.

6.6.2.3A.5.4.2	Test procedure

1.	Configure SCCs according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCCs as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.6.2.3A.5.4.3

3.	SS activates SCCs by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.6.2.3A.5.4.1-1 all PCC and SCCs. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control “up” commands in every uplink scheduling information to the UE until the UE transmits at PUMAX level.

6.	Measure the mean power over all component carriers of the UE in the CA configuration of the radio access mode according to the test configuration, which shall meet the requirements described in Table 6.2.3A.5.5-1. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

7.	Measure the rectangular filtered mean power for CA E-UTRA in the first band of the CA configuration using a rms detector.

8.	Measure the rectangular filtered mean power of the first CA E-UTRA adjacent channel on both lower and upper side of the CA E-UTRA channel in the first band of the CA configuration using a rms detector, respectively.

9.	Measure the RRC filtered mean power of the first and the second UTRA adjacent channel on both lower and upper side of the CA E-UTRA channel in the first band of the CA configuration using a rms detector, respectively.

10.	Calculate the ratio of the power between the values measured in step 7 over step 8 for CA E-UTRAACLR.

11.	Calculate the ratio of the power between the values measured in step 7 over step 9 for UTRAACLR1, UTRAACLR2.

12. Repeat steps 6 to 11 for the second band of the CA configuration.

6.6.2.3A.5.5	Test Requirements

6.6.2.3A.5.5.1	Test requirements of UTRA for CA

If the measured UTRA channel power is greater than -50dBm then the measured UTRAACLR1 and UTRAACLR2, derived in step 11, shall be higher than the limits in table 6.6.2.3A.5.5.1-1.

Table 6.6.2.3A.5.5.1-1: UTRA UE ACLR for CA

|  | CA bandwidth class / UTRAACLR1/2 / measurement bandwidth |
| --- | --- |
|  | CA bandwidth class D |
| UTRAACLR1 | 32.2 dB |
| Adjacent channel centre frequency offset (in MHz) | + BWChannel_CA /2 + BWUTRA/2 / - BWChannel_CA / 2 - BWUTRA/2 |
| UTRAACLR2 | 35.2 dB |
| Adjacent channel centre frequency offset (in MHz) | + BWChannel_CA /2 + 3*BWUTRA/2 / - BWChannel_CA /2 – 3*BWUTRA/2 |
| CA E-UTRA channel Measurement bandwidth | BWChannel_CA - 2* BWGB |
| UTRA 5MHz channel Measurement bandwidth (Note 1) | 3.84 MHz |
| UTRA 1.6MHz channel measurement bandwidth (Note 2) | 1.28 MHz |
| Note 1: Applicable for E-UTRA FDD co-existence with UTRA FDD in paired spectrum. Note 2: Applicable for E-UTRA TDD co-existence with UTRA TDD in unpaired spectrum. |  |

6.6.2.3A.5.5.2	Test requirements of CA E-UTRA

The measured UE mean power in the channel bandwidth as specified in clause 5.4.2A, derived in step 6, shall fulfil requirements in Table 6.2.3A.5.5-1 and if the measured adjacent channel power is greater than –50 dBm then the measured CA E-UTRAACLR, derived in step 10 shall be higher than the limits in table 6.6.2.3A.5.5.2-1.

Table 6.6.2.3A.5.5.2-1: CA E-UTRA ACLR

|  | CA bandwidth class / CA E-UTRAACLR / Measurement bandwidth |
| --- | --- |
|  | CA bandwidth class D |
| CA E-UTRAACLR | 29.2 dB |
| CA E-UTRA channel Measurement bandwidth | BWChannel_CA - 2* BWGB |
| Adjacent channel centre frequency offset (in MHz) | + BWChannel_CA / - BWChannel_CA |

#### 6.6.2.3B	Adjacent Channel Leakage power Ratio for UL-MIMO

Editor’s Note:

- Test applicability specification needs to be updated for not applicable bands of UTRAACLR

##### 6.6.2.3B.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR) for UL-MIMO.

##### 6.6.2.3B.2	Test applicability

This test case applies to all types of E-UTRA UE release 10 and forward that support UL-MIMO.

The test of UTRAACLR doesn’t apply on power class 3 UE operating in Band 7, 12, 13, 17, 20, 24, 27, 30, 33, 35, 36, 37, 38, 40, 43, 44, 45, 47, 48, 50, 51, 52, 53, 54, 68, 70, 71, 85, 87, 88, 106 and Scell operation in Band 46, 49.

##### 6.6.2.3B.3	Minimum conformance requirements

For UE with multiple transmit antenna connectors, the requirements for ACLR are specified at each transmit antenna connector. ACLR requirements are specified for two scenarios for an adjacent E -UTRAACLR and UTRAACLR1/2 as shown in Figure 6.6.2.3B.3-1.

Figure 6.6.2.3B.3-1: Adjacent Channel Leakage Power Ratio requirements

6.6.2.3B.3.1	Minimum conformance requirements for E-UTRA

E-UTRA ACLR (E-UTRAACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency at nominal channel spacing. The assigned E-UTRA channel power and adjacent E-UTRA channel power are measured with rectangular filters with measurement bandwidth specified in Table 6.6.2.3B.3.1-1.

For UEs with two transmit antenna connectors in closed-loop spatial multiplexing scheme ,, the requirements in Table 6.6.2.3B.3.1-1 apply to each transmit antenna connector with the UL-MIMO configurations specified in Table 6.2.2B.3-2.

If the measured adjacent channel power is greater than -50dBm then the E-UTRAACLR shall be higher than the valued specified in Table 6.6.2.3B.3.1-1.

Table 6.6.2.3B.3.1-1: General requirements for E-UTRAACLR

|  | Channel bandwidth / E-UTRAACLR1 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | 30 dB | 30 dB | 30 dB | 30 dB | 30 dB | 30 dB |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |

The normative reference for this requirement is TS 36.101 clause 6.6.2B.

6.6.2.3B.3.2	Minimum conformance requirements for UTRA

UTRA ACLR (UTRAACLR) is the ratio of the filtered mean power centred on the assigned E-UTRA channel frequency to the filtered mean power centred on an adjacent UTRA channel frequency.

UTRA ACLR is specified for both the first UTRA adjacent channel (UTRAACLR1) and the 2nd UTRA adjacent channel (UTRAACLR2).The UTRA channel power is measured with a RRC bandwidth filter with roll-off factor = 0.22. The assigned E-UTRA channel power is measured with a rectangular filter with measurement bandwidth specified in Table 6.6.2.3B.3.2-1.

For UEs with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the requirements in Table 6.6.2.3B.3.2-1 apply to each transmit antenna connector with the UL-MIMO configurations specified in Table 6.2.2B.3-2.

If the measured UTRA channel power is greater than -50dBm then the UTRAACLR1, and UTRAACLR2 shall be higher than the valued specified in Table 6.6.2.3B.3.2-1.

UTRAACLR is not applicable to the power class 3 UE operating in Band 7, 12, 13, 17, 20, 24, 27, 30, 33, 35, 36, 37, 38, 40, 43, 44, 45, 47, 48, 50, 51, 52, 53, 54, 68, 70, 71, 85, 87, 88, 106 and Scell operation in Band 46, 49.

Table 6.6.2.3B.3.2-1: General requirements for UTRAACLR1/2

|  | Channel bandwidth / E-UTRAACLR1/2 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | 33 dB | 33 dB | 33 dB | 33 dB | 33 dB | 33 dB |
| Adjacent channel centre frequency offset (in MHz) | 0.7+BWUTRA/2 / -0.7-BWUTRA/2 | 1.5+BWUTRA/2 / -1.5-BWUTRA/2 | 2.5+BWUTRA/2 / -2.5-BWUTRA/2 | 5+BWUTRA/2 / -5-BWUTRA/2 | 7.5+BWUTRA/2 / -7.5-BWUTRA/2 | 10+BWUTRA/2 / -10-BWUTRA/2 |
| UTRAACLR2 | - | - | 36 dB | 36 dB | 36 dB | 36 dB |
| Adjacent channel centre frequency offset (in MHz) | - | - | 2.5+3*BWUTRA/2 / -2.5-3*BWUTRA/2 | 5+3*BWUTRA/2 / -5-3*BWUTRA/2 | 7.5+3*BWUTRA/2 / -7.5-3*BWUTRA/2 | 10+3*BWUTRA/2 / -10-3*BWUTRA/2 |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UTRA 5MHz channel Measurement bandwidth1 | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz |
| UTRA 1.6MHz channel measurement bandwidth2 | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz |
| Note 1: Applicable for E-UTRA FDD co-existence with UTRA FDD in paired spectrum. Note 2: Applicable for E-UTRA TDD co-existence with UTRA TDD in unpaired spectrum. |  |  |  |  |  |  |

The normative reference for this requirement is TS 36.101 clause 6.6.2B.

##### 6.6.2.3B.4	Test description

6.6.2.3B.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.3B.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in AnnexeA.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.3B.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] clause 4.1) |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies (as specified in TS 36.508 [7] clause 4.3.1) |  |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] clause 4.3.1) |  |  |  | Lowest, 5MHz, 10MHz, Highest |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |  |
| Ch BW | Mod'n | RB allocation |  | Mod'n | RB allocation |  |
|  |  | FDD | TDD |  | FDD | TDD |
| 1.4MHz | N/A for ACLR testing |  |  | QPSK | 6 | 6 |
| 1.4MHz |  |  |  | QPSK | 5 | 5 |
| 1.4MHz |  |  |  | 16QAM | 6 | 6 |
| 1.4MHz |  |  |  | 16QAM | 5 | 5 |
| 3MHz |  |  |  | QPSK | 15 | 15 |
| 3MHz |  |  |  | QPSK | 4 | 4 |
| 3MHz |  |  |  | 16QAM | 15 | 15 |
| 3MHz |  |  |  | 16QAM | 4 | 4 |
| 5MHz |  |  |  | QPSK | 25 | 25 |
| 5MHz |  |  |  | QPSK | 8 | 8 |
| 5MHz |  |  |  | 16QAM | 25 | 25 |
| 5MHz |  |  |  | 16QAM | 8 | 8 |
| 10MHz |  |  |  | QPSK | 50 | 50 |
| 10MHz |  |  |  | QPSK | 12 | 12 |
| 10MHz |  |  |  | 16QAM | 50 (Note 3) | 50 (Note 3) |
| 10MHz |  |  |  | 16QAM | 12 | 12 |
| 15MHz |  |  |  | QPSK | 75 | 75 |
| 15MHz |  |  |  | QPSK | 16 | 16 |
| 15MHz |  |  |  | 16QAM | 75 (Note 3) | 75 (Note 3) |
| 15MHz |  |  |  | 16QAM | 16 | 16 |
| 20MHz |  |  |  | QPSK | 100 | 100 |
| 20MHz |  |  |  | QPSK | 18 | 18 |
| 20MHz |  |  |  | 16QAM | 100 (Note 3) | 100 (Note 3) |
| 20MHz |  |  |  | 16QAM | 18 | 18 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The RBstart of partial RB allocation shall be RB#0 and RB# (max + 1 - RB allocation) of the channel bandwidth. Note 3: Applies only for UE-Categories [FFS]. |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure A.28.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] clause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.3B.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.3B.4.3.

6.6.2.3B.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 4 for C_RNTI to schedule the UL RMC according to Table 6.6.2.3B.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the sum of the mean power at each antenna connector of UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in clause 6.2.3B.5. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test.

4.	Measure the rectangular filtered mean power for E-UTRA at each antenna connector of UE using a rms detector.

5.	Measure the rectangular filtered mean power of the first E-UTRA adjacent channel at each antenna connector of UE using a rms detector.

6.	Measure the RRC filtered mean power of the first and the second UTRA adjacent channel at each antenna connector of UE using a rms detector.

7.	Calculate the ratio of the power between the values measured in step 4 over step 5 for E-UTRAACLR.

8.	Calculated the ratio of the power between the values measured in step 4 over step 6 for UTRAACLR1, UTRAACLR2.

6.6.2.3B.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6.

##### 6.6.2.3B.5	Test requirement

6.6.2.3B.5.1	Test requirements E-UTRA

-	The measured sum of mean power at each transmit antenna connector for UE in the channel bandwidth, derived in step 3, shall fulfil requirements in clause 6.2.3B.5 as appropriate,

and

-	The requirements apply to each transmit antenna connector.

-	if the measured adjacent channel power is greater than -50 dBm then the measured E-UTRAACLR, derived in step 7, shall be higher than the limits in table 6.6.2.3B.5.1-1.

Table 6.6.2.3B.5.1-1: E-UTRA UE ACLR

|  | Channel bandwidth / E-UTRAACLR1 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UE channel | +1.4 MHz or -1.4 MHz | +3 MHz or -3 MHz | +5MHz or -5MHz | +10MHz or -10MHz | +15MHz or -15MHz | +20MHz or -20MHz |

6.6.2.3B.5.2	Test requirements UTRA

The requirements apply to each transmit antenna connector.

If the measured UTRA channel power is greater than -50dBm then the measured UTRAACLR1, UTRAACLR2, derived in step 8, shall be higher than the limits in table 6.6.2.3B.5.2-1.

Table 6.6.2.3B.5.2-1: UTRA UE ACLR

|  | Channel bandwidth / E-UTRAACLR1/2 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| UTRAACLR1 | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB |
| Adjacent channel centre frequency offset (in MHz) | 0.7+BWUTRA/2 / -0.7-BWUTRA/2 | 1.5+BWUTRA/2 / -1.5-BWUTRA/2 | 2.5+BWUTRA/2 / -2.5-BWUTRA/2 | 5+BWUTRA/2 / -5-BWUTRA/2 | 7.5+BWUTRA/2 / -7.5-BWUTRA/2 | 10+BWUTRA/2 / -10-BWUTRA/2 |
| UTRAACLR2 | - | - | 35.2 dB | 35.2 dB | 35.2 dB | 35.2 dB |
| Adjacent channel centre frequency offset (in MHz) | - | - | 2.5+3*BWUTRA/2 / -2.5-3*BWUTRA/2 | 5+3*BWUTRA/2 / -5-3*BWUTRA/2 | 7.5+3*BWUTRA/2 / -7.5-3*BWUTRA/2 | 10+3*BWUTRA/2 / -10-3*BWUTRA/2 |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UTRA 5MHz channel Measurement bandwidth1 | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz |
| UTRA 1.6MHz channel measurement bandwidth2 | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz |
| Note 1: Applicable for E-UTRA FDD co-existence with UTRA FDD in paired spectrum. Note 2: Applicable for E-UTRA TDD co-existence with UTRA TDD in unpaired spectrum. Note 3: BWUTRA for UTRA FDD is 5MHz and for UTRA TDD is 1.6MHz. |  |  |  |  |  |  |

#### 6.6.2.3E	Adjacent Channel Leakage power Ratio for UE category 0

##### 6.6.2.3E.1	Test purpose

Same test purpose as in clause 6.6.2.3.1.

##### 6.6.2.3E.2	Test applicability

This test case applies to all types of E-UTRA UE release 12 and forward of UE category 0.

##### 6.6.2.3E.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.6.2.3.3.

##### 6.6.2.3E.4	Test description

6.6.2.3E.4.1	Initial conditions

Same initial conditions as in clause 6.6.2.3.4.1 with following exceptions:

-	Instead of Table 6.6.2.3.4.1-1  use Table 6.6.2.3E.4.1-1.

-	Connect SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.3 using only main UE Tx/Rx antenna.

Table 6.6.2.3E.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Lowest, 5MHz, 10MHz, Highest |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |  |
| Ch BW | Mod'n | RB allocation |  | Mod'n | RB allocation |  |
|  |  | FDD | TDD |  | FDD and HD-FDD | TDD |
| 1.4MHz | N/A for ACLR testing |  |  | QPSK | 6 | 6 |
| 1.4MHz |  |  |  | QPSK | 5 | 5 |
| 1.4MHz |  |  |  | 16QAM | 5 | 5 |
| 3MHz |  |  |  | QPSK | 15 | 15 |
| 3MHz |  |  |  | QPSK | 4 | 4 |
| 3MHz |  |  |  | 16QAM | 5 | 5 |
| 3MHz |  |  |  | 16QAM | 4 | 4 |
| 5MHz |  |  |  | QPSK | 25 | 25 |
| 5MHz |  |  |  | QPSK | 8 | 8 |
| 5MHz |  |  |  | 16QAM | 5 | 5 |
| 10MHz |  |  |  | QPSK | 25 | 25 |
| 10MHz |  |  |  | QPSK | 12 | 12 |
| 10MHz |  |  |  | 16QAM | 5 | 5 |
| 15MHz |  |  |  | QPSK | 36 | 36 |
| 15MHz |  |  |  | QPSK | 16 | 16 |
| 15MHz |  |  |  | 16QAM | 5 | 5 |
| 20MHz |  |  |  | QPSK | 36 | 36 |
| 20MHz |  |  |  | QPSK | 18 | 18 |
| 20MHz |  |  |  | 16QAM | 5 | 5 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The RBstart of partial RB allocation shall be RB#0 and RB# (max + 1 - RB allocation) of the channel bandwidth. |  |  |  |  |  |  |

6.6.2.3E.4.2	Test procedure

Same test procedure as in clause 6.6.2.3.4.2 with following exception for HD-FDD:

-	In step 3, slots with transient periods are not under test. Half-duplex guard subframe are not under test.

6.6.2.3E.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6.

##### 6.6.2.3E.5	Test requirement

6.6.2.3E.5.1	Test requirements E-UTRA

-	The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.3E.5-1 as appropriate,

and

-	if the measured adjacent channel power is greater than –50 dBm then the measured E-UTRAACLR, derived in step 7, shall be higher than the limits in table 6.6.2.3E.5.1-1.

Table 6.6.2.3E.5.1-1: E-UTRA UE ACLR

|  | Channel bandwidth / E-UTRAACLR1 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UE channel | +1.4 MHz or -1.4 MHz | +3 MHz or -3 MHz | +5MHz or -5MHz | +10MHz or -10MHz | +15MHz or -15MHz | +20MHz or -20MHz |

Test requirements UTRA

If the measured UTRA channel power is greater than –50dBm then the measured UTRAACLR1, UTRAACLR2, derived in step 8, shall be higher than the limits in table 6.6.2.3E.5.2-1.

Table 6.6.2.3E.5.2-1: UTRA UE ACLR

|  | Channel bandwidth / UTRAACLR1/2 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| UTRAACLR1 | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB |
| Adjacent channel centre frequency offset (in MHz) | 0.7+BWUTRA/2 / -0.7-BWUTRA/2 | 1.5+BWUTRA/2 / -1.5-BWUTRA/2 | 2.5+BWUTRA/2 / -2.5-BWUTRA/2 | 5+BWUTRA/2 / -5-BWUTRA/2 | 7.5+BWUTRA/2 / -7.5-BWUTRA/2 | 10+BWUTRA/2 / -10-BWUTRA/2 |
| UTRAACLR2 | - | - | 35.2 dB | 35.2 dB | 35.2 dB | 35.2 dB |
| Adjacent channel centre frequency offset (in MHz) | - | - | 2.5+3*BWUTRA/2 / -2.5-3*BWUTRA/2 | 5+3*BWUTRA/2 / -5-3*BWUTRA/2 | 7.5+3*BWUTRA/2 / -7.5-3*BWUTRA/2 | 10+3*BWUTRA/2 / -10-3*BWUTRA/2 |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UTRA 5MHz channel Measurement bandwidth1 | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz |
| UTRA 1.6MHz channel measurement bandwidth2 | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz |
| Note 1: Applicable for E-UTRA FDD co-existence with UTRA FDD in paired spectrum. Note 2: Applicable for E-UTRA TDD co-existence with UTRA TDD in unpaired spectrum. Note 3: BWUTRA for UTRA FDD is 5MHz and for UTRA TDD is 1.6MHz. |  |  |  |  |  |  |

#### 6.6.2.3EA	Adjacent Channel Leakage power Ratio for UE category M1

##### 6.6.2.3EA.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR).

##### 6.6.2.3EA.2	Test applicability

This test case applies to all types of E-UTRA UE release 13 and forward of UE category M1.

##### 6.6.2.3EA.3	Minimum conformance requirements

ACLR requirements are specified for two scenarios for an adjacent E -UTRAACLR and UTRAACLR1/2 as shown in Figure 6.6.2.3EA.3-1.

Figure 6.6.2.3EA.3-1: Adjacent Channel Leakage Power Ratio requirements

6.6.2.3EA.3.1	Minimum conformance requirements for E-UTRA

E-UTRA ACLR (E-UTRAACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency at nominal channel spacing. The assigned E-UTRA channel power and adjacent E-UTRA channel power are measured with rectangular filters with measurement bandwidth specified in Table 6.6.2.3EA.3.1-1.

If the measured adjacent channel power is greater than –50dBm then the E-UTRAACLR shall be higher than the valued specified in Table 6.6.2.3EA.3.1-1.

Table 6.6.2.3EA.3.1-1: General requirements for E-UTRAACLR

|  | Channel bandwidth / E-UTRAACLR1 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | 30 dB | 30 dB | 30 dB | 30 dB | 30 dB | 30 dB |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |

The normative reference for this requirement is TS 36.101 [2] subclause 6.6.2.3.1.

6.6.2.3EA.3.2	 Minimum conformance requirements for UTRA

UTRA ACLR (UTRAACLR) is the ratio of the filtered mean power centred on the assigned E-UTRA channel frequency to the filtered mean power centred on an adjacent UTRA channel frequency.

UTRA ACLR is specified for both the first UTRA adjacent channel (UTRAACLR1) and the 2nd UTRA adjacent channel (UTRAACLR2).The UTRA channel power is measured with a RRC bandwidth filter with roll-off factor = 0.22. The assigned E-UTRA channel power is measured with a rectangular filter with measurement bandwidth specified in Table 6.6.2.3EA.3.2-1.

If the measured UTRA channel power is greater than –50dBm then the UTRAACLR1, and UTRAACLR2 shall be higher than the valued specified in Table 6.6.2.3EA.3.2-1.

Table 6.6.2.3EA.3.2-1: General requirements for UTRAACLR1/2

|  | Channel bandwidth / UTRAACLR1/2 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| UTRAACLR1 | 33 dB | 33 dB | 33 dB | 33 dB | 33 dB | 33 dB |
| Adjacent channel centre frequency offset (in MHz) | 0.7+BWUTRA/2 / -0.7-BWUTRA/2 | 1.5+BWUTRA/2 / -1.5-BWUTRA/2 | 2.5+BWUTRA/2 / -2.5-BWUTRA/2 | 5+BWUTRA/2 / -5-BWUTRA/2 | 7.5+BWUTRA/2 / -7.5-BWUTRA/2 | 10+BWUTRA/2 / -10-BWUTRA/2 |
| UTRAACLR2 | - | - | 36 dB | 36 dB | 36 dB | 36 dB |
| Adjacent channel centre frequency offset (in MHz) | - | - | 2.5+3*BWUTRA/2 / -2.5-3*BWUTRA/2 | 5+3*BWUTRA/2 / -5-3*BWUTRA/2 | 7.5+3*BWUTRA/2 / -7.5-3*BWUTRA/2 | 10+3*BWUTRA/2 / -10-3*BWUTRA/2 |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UTRA 5MHz channel Measurement bandwidth1 | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz |
| UTRA 1.6MHz channel measurement bandwidth2 | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz |
| Note 1: Applicable for E-UTRA FDD co-existence with UTRA FDD in paired spectrum. Note 2: Applicable for E-UTRA TDD co-existence with UTRA TDD in unpaired spectrum. |  |  |  |  |  |  |

The normative reference for this requirement is TS 36.101 subclause 6.6.2.3.2.

##### 6.6.2.3EA.4	Test description

6.6.2.3EA.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in clause 5.2E. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.3EA.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in AnnexeA.2. Configurations of PDSCH and MPDCCH before measurement are specified in Annex C.2.

Table 6.6.2.3EA.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  | Lowest, 5MHz, 10MHz, Highest |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A for ACLR test case | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| Low range, Mid range |  |  |  |  |  |
| 1 | 1.4MHz |  | QPSK | 2 | 0 |
| 2 | 1.4MHz |  | QPSK | 5 | 0 |
| 3 | 1.4MHz |  | QPSK | 6 | 0 |
| 4 | 1.4MHz |  | 16QAM | 2 | 0 |
| 5 | 1.4MHz |  | 16QAM | 5 | 0 |
| 6 | 3MHz |  | QPSK | 2 | 0 |
| 7 | 3MHz |  | QPSK | 5 | 0 |
| 8 | 3MHz |  | QPSK | 6 | 0 |
| 9 | 3MHz |  | 16QAM | 2 | 0 |
| 10 | 3MHz |  | 16QAM | 5 | 0 |
| 11 | 5MHz |  | QPSK | 6 | 0 |
| 12 (Note 4) | 5MHz |  | 16QAM | 1 | 0 |
| 13 | 5MHz |  | 16QAM | 3 | 0 |
| 14 | 5MHz |  | 16QAM | 5 | 0 |
| 15 (Note 4) | 10MHz |  | QPSK | 4 | 0 |
| 16 | 10MHz |  | QPSK | 6 | 0 |
| 17 (Note 4) | 10MHz |  | 16QAM | 3 | 0 |
| 18 | 10MHz |  | 16QAM | 5 | 0 |
| 19 | 15MHz |  | QPSK | 6 | 0 |
| 20 | 15MHz |  | 16QAM | 5 | 0 |
| 21 | 20MHz |  | 16QAM | 5 | 0 |
| High range |  |  |  |  |  |
| 1 | 1.4MHz |  | QPSK | 2 | 0 |
| 2 | 1.4MHz |  | QPSK | 5 | 0 |
| 3 | 1.4MHz |  | QPSK | 6 | 0 |
| 4 | 1.4MHz |  | 16QAM | 2 | 0 |
| 5 | 1.4MHz |  | 16QAM | 5 | 0 |
| 6 | 3MHz |  | QPSK | 2 | 1 |
| 7 | 3MHz |  | QPSK | 5 | 1 |
| 8 | 3MHz |  | QPSK | 6 | 1 |
| 9 | 3MHz |  | 16QAM | 2 | 1 |
| 10 | 3MHz |  | 16QAM | 5 | 1 |
| 11 | 5MHz |  | QPSK | 6 | 3 |
| 12 (Note 4) | 5MHz |  | 16QAM | 1 | 3 |
| 13 | 5MHz |  | 16QAM | 3 | 3 |
| 14 | 5MHz |  | 16QAM | 5 | 3 |
| 15 (Note 4) | 10MHz |  | QPSK | 4 | 7 |
| 16 | 10MHz |  | QPSK | 6 | 7 |
| 17 (Note 4) | 10MHz |  | 16QAM | 3 | 7 |
| 18 | 10MHz |  | 16QAM | 5 | 7 |
| 19 | 15MHz |  | QPSK | 6 | 11 |
| 20 | 15MHz |  | 16QAM | 5 | 11 |
| 21 | 20MHz |  | 16QAM | 5 | 15 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4 Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 - RB allocation) of the narrowband. Note 4: Only for power class 3 UE |  |  |  |  |  |

Table 6.6.2.3EA.4.1-2: Test Configuration Table, subPRB allocation

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 5MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| Low range, Mid range |  |  |  |  |  |
| 1 | 5MHz |  | QPSK | ½ (6 SCs) | 0 |
| High range |  |  |  |  |  |
| 2 | 5MHz |  | QPSK | ½ (6 SCs) | 3 |
| Note 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.7. Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The SCstart shall be SC#0 and SC# (72 – RB allocation) of the narrowband, when RB allocation is defined as #SCs |  |  |  |  |  |

1.	Connect SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.3EA.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [7] clause 5.2A.2AA. Message contents are defined in clause 6.6.2.3EA.4.3.

7.	For UE supporting subPRB allocation, repeat step 1-6 with UL RMC according to Table 6.6.2.3EA.4.1-2.

6.6.2.3EA.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via MPDCCH DCI format 6-0A for C_RNTI to schedule the UL RMC according to Table 6.6.2.3EA.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Table 6.2.3EA.5-1 to 6.2.3EA.5-7a . The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test. For HD-FDD slots with transient periods and Half-duplex guard subframe are not under test.

4.	Measure the rectangular filtered mean power for E-UTRA using a rms detector.

5.	Measure the rectangular filtered mean power of the first E-UTRA adjacent channel on both lower and upper side of the E-UTRA channel using a rms detector, respectively.

6.	Measure the RRC filtered mean power of the first and the second UTRA adjacent channel on both lower and upper side of the E-UTRA channel using a rms detector, respectively.

7.	Calculate the ratios of the power between the values measured in step 4 over step 5 for lower and upper E-UTRAACLR, respectively.

8.	Calculated the ratios of the power between the values measured in step 4 over step 6 for lower and upper UTRAACLR1, UTRAACLR2, respectively.

6.6.2.3EA.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6.

##### 6.6.2.3EA.5	Test requirement

6.6.2.3EA.5.1	Test requirements E-UTRA

-	The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in [Table 6.2.3EA.5-1 to 6.2.3EA.5-7, as appropriate,

and

-	if the measured adjacent channel power is greater than –50 dBm then the measured E-UTRAACLR, derived in step 7, shall be higher than the limits in table 6.6.2.3EA.5.1-1.

Table 6.6.2.3EA.5.1-1: E-UTRA UE ACLR

|  | Channel bandwidth / E-UTRAACLR1 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UE channel | +1.4 MHz or -1.4 MHz | +3 MHz or -3 MHz | +5MHz or -5MHz | +10MHz or -10MHz | +15MHz or -15MHz | +20MHz or -20MHz |

6.6.2.3EA.5.2	Test requirements UTRA

If the measured UTRA channel power is greater than –50dBm then the measured UTRAACLR1, UTRAACLR2, derived in step 8, shall be higher than the limits in table 6.6.2.3EA.5.2-1.

Table 6.6.2.3EA.5.2-1: UTRA UE ACLR

|  | Channel bandwidth / UTRAACLR1/2 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| UTRAACLR1 | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB |
| Adjacent channel centre frequency offset (in MHz) | 0.7+BWUTRA/2 / -0.7-BWUTRA/2 | 1.5+BWUTRA/2 / -1.5-BWUTRA/2 | 2.5+BWUTRA/2 / -2.5-BWUTRA/2 | 5+BWUTRA/2 / -5-BWUTRA/2 | 7.5+BWUTRA/2 / -7.5-BWUTRA/2 | 10+BWUTRA/2 / -10-BWUTRA/2 |
| UTRAACLR2 | - | - | 35.2 dB | 35.2 dB | 35.2 dB | 35.2 dB |
| Adjacent channel centre frequency offset (in MHz) | - | - | 2.5+3*BWUTRA/2 / -2.5-3*BWUTRA/2 | 5+3*BWUTRA/2 / -5-3*BWUTRA/2 | 7.5+3*BWUTRA/2 / -7.5-3*BWUTRA/2 | 10+3*BWUTRA/2 / -10-3*BWUTRA/2 |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UTRA 5MHz channel Measurement bandwidth1 | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz |
| UTRA 1.6MHz channel measurement bandwidth2 | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz |
| Note 1: Applicable for E-UTRA FDD co-existence with UTRA FDD in paired spectrum. Note 2: Applicable for E-UTRA TDD co-existence with UTRA TDD in unpaired spectrum. Note 3: BWUTRA for UTRA FDD is 5MHz and for UTRA TDD is 1.6MHz. |  |  |  |  |  |  |

#### 6.6.2.3EB	Adjacent Channel Leakage power Ratio for UE category 1bis

##### 6.6.2.3EB.1	Test purpose

Same test purpose as in clause 6.6.2.3.1.

##### 6.6.2.3EB.2	Test applicability

This test case applies to all types of E-UTRA UE release 13 and forward of UE category 1bis.

##### 6.6.2.3EB.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.6.2.3.3.

##### 6.6.2.3EB.4	Test description

6.6.2.3EB.4.1	Initial conditions

Same initial conditions as in clause 6.6.2.3.4.1 with following exceptions:

-	Connect SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.3 using only main UE Tx/Rx antenna.

6.6.2.3EB.4.2	Test procedure

Same test procedure as in clause 6.6.2.3.4.2

6.6.2.3EB.4.3	Message contents

Message contents same as clause 6.6.2.3.4.3.

##### 6.6.2.3EB.5	Test requirement

6.6.2.3EB.5.1	Test requirements E-UTRA

-	The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.3EB.5-1 as appropriate,

and

-	if the measured adjacent channel power is greater than –50 dBm then the measured E-UTRAACLR, derived in step 7, shall be higher than the limits in table 6.6.2.3EB.5.1-1.

Table 6.6.2.3EB.5.1-1: E-UTRA UE ACLR

|  | Channel bandwidth / E-UTRAACLR1 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UE channel | +1.4 MHz or -1.4 MHz | +3 MHz or -3 MHz | +5MHz or -5MHz | +10MHz or -10MHz | +15MHz or -15MHz | +20MHz or -20MHz |

6.6.2.3EB.5.2	Test requirements UTRA

If the measured UTRA channel power is greater than –50dBm then the measured UTRAACLR1, UTRAACLR2, derived in step 8, shall be higher than the limits in table 6.6.2.3EB.5.2-1.

Table 6.6.2.3EB.5.2-1: UTRA UE ACLR

|  | Channel bandwidth / UTRAACLR1/2 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| UTRAACLR1 | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB |
| Adjacent channel centre frequency offset (in MHz) | 0.7+BWUTRA/2 / -0.7-BWUTRA/2 | 1.5+BWUTRA/2 / -1.5-BWUTRA/2 | 2.5+BWUTRA/2 / -2.5-BWUTRA/2 | 5+BWUTRA/2 / -5-BWUTRA/2 | 7.5+BWUTRA/2 / -7.5-BWUTRA/2 | 10+BWUTRA/2 / -10-BWUTRA/2 |
| UTRAACLR2 | - | - | 35.2 dB | 35.2 dB | 35.2 dB | 35.2 dB |
| Adjacent channel centre frequency offset (in MHz) | - | - | 2.5+3*BWUTRA/2 / -2.5-3*BWUTRA/2 | 5+3*BWUTRA/2 / -5-3*BWUTRA/2 | 7.5+3*BWUTRA/2 / -7.5-3*BWUTRA/2 | 10+3*BWUTRA/2 / -10-3*BWUTRA/2 |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UTRA 5MHz channel Measurement bandwidth1 | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz |
| UTRA 1.6MHz channel measurement bandwidth2 | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz |
| Note 1: Applicable for E-UTRA FDD co-existence with UTRA FDD in paired spectrum. Note 2: Applicable for E-UTRA TDD co-existence with UTRA TDD in unpaired spectrum. Note 3: BWUTRA for UTRA FDD is 5MHz and for UTRA TDD is 1.6MHz. |  |  |  |  |  |  |

#### 6.6.2.3EC	Adjacent Channel Leakage power Ratio for UE category M2

##### 6.6.2.3EC.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR).

##### 6.6.2.3EC.2	Test applicability

This test case applies to all types of E-UTRA UE release 14 and forward of UE category M2.

##### 6.6.2.3EC.3	Minimum conformance requirements

ACLR requirements are specified for two scenarios for an adjacent E -UTRAACLR and UTRAACLR1/2 as shown in Figure 6.6.2.3EC.3-1.

Figure 6.6.2.3EC.3-1: Adjacent Channel Leakage Power Ratio requirements

6.6.2.3EC.3.1	Minimum conformance requirements for E-UTRA

E-UTRA ACLR (E-UTRAACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency at nominal channel spacing. The assigned E-UTRA channel power and adjacent E-UTRA channel power are measured with rectangular filters with measurement bandwidth specified in Table 6.6.2.3EC.3.1-1.

If the measured adjacent channel power is greater than –50dBm then the E-UTRAACLR shall be higher than the valued specified in Table 6.6.2.3EC.3.1-1.

Table 6.6.2.3EC.3.1-1: General requirements for E-UTRAACLR

|  | Channel bandwidth / E-UTRAACLR1 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | 30 dB | 30 dB | 30 dB | 30 dB | 30 dB | 30 dB |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |

The normative reference for this requirement is TS 36.101 [2] subclause 6.6.2.3.1.

6.6.2.3EC.3.2	 Minimum conformance requirements for UTRA

UTRA ACLR (UTRAACLR) is the ratio of the filtered mean power centred on the assigned E-UTRA channel frequency to the filtered mean power centred on an adjacent UTRA channel frequency.

UTRA ACLR is specified for both the first UTRA adjacent channel (UTRAACLR1) and the 2nd UTRA adjacent channel (UTRAACLR2).The UTRA channel power is measured with a RRC bandwidth filter with roll-off factor = 0.22. The assigned E-UTRA channel power is measured with a rectangular filter with measurement bandwidth specified in Table 6.6.2.3EC.3.2-1.

If the measured UTRA channel power is greater than –50dBm then the UTRAACLR1, and UTRAACLR2 shall be higher than the valued specified in Table 6.6.2.3EC.3.2-1.

Table 6.6.2.3EC.3.2-1: General requirements for UTRAACLR1/2

|  | Channel bandwidth / UTRAACLR1/2 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| UTRAACLR1 | 33 dB | 33 dB | 33 dB | 33 dB | 33 dB | 33 dB |
| Adjacent channel centre frequency offset (in MHz) | 0.7+BWUTRA/2 / -0.7-BWUTRA/2 | 1.5+BWUTRA/2 / -1.5-BWUTRA/2 | 2.5+BWUTRA/2 / -2.5-BWUTRA/2 | 5+BWUTRA/2 / -5-BWUTRA/2 | 7.5+BWUTRA/2 / -7.5-BWUTRA/2 | 10+BWUTRA/2 / -10-BWUTRA/2 |
| UTRAACLR2 | - | - | 36 dB | 36 dB | 36 dB | 36 dB |
| Adjacent channel centre frequency offset (in MHz) | - | - | 2.5+3*BWUTRA/2 / -2.5-3*BWUTRA/2 | 5+3*BWUTRA/2 / -5-3*BWUTRA/2 | 7.5+3*BWUTRA/2 / -7.5-3*BWUTRA/2 | 10+3*BWUTRA/2 / -10-3*BWUTRA/2 |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UTRA 5MHz channel Measurement bandwidth1 | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz |
| UTRA 1.6MHz channel measurement bandwidth2 | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz |
| Note 1: Applicable for E-UTRA FDD co-existence with UTRA FDD in paired spectrum. Note 2: Applicable for E-UTRA TDD co-existence with UTRA TDD in unpaired spectrum. |  |  |  |  |  |  |

The normative reference for this requirement is TS 36.101 subclause 6.6.2.3.2.

##### 6.6.2.3EC.4	Test description

6.6.2.3EC.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in clause 5.2E. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.3EC.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in AnnexeA.2. Configurations of PDSCH and MPDCCH before measurement are specified in Annex C.2.

Table 6.6.2.3EC.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Lowest, 5MHz, 10MHz, Highest |  |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |  |
| Configuration ID | Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |  |  |
|  |  |  |  | FDD and HD-FDD | TDD | Narrowband index (Note 1) |
| Low range, Mid range |  |  |  |  |  |  |
| 1 | 1.4MHz |  | QPSK | 2 | 2 | 0 |
| 2 | 1.4MHz |  | QPSK | 5 | 5 | 0 |
| 3 | 1.4MHz |  | QPSK | 6 | 6 | 0 |
| 4 | 1.4MHz |  | 16QAM | 2 | 2 | 0 |
| 5 | 1.4MHz |  | 16QAM | 6 | 6 | 0 |
| 6 | 3MHz |  | QPSK | 2 | 2 | 0 |
| 7 | 3MHz |  | QPSK | 5 | 5 | 0 |
| 8 | 3MHz |  | QPSK | 12 | 12 | 0 |
| 9 | 3MHz |  | 16QAM | 2 | 2 | 0 |
| 10 | 3MHz |  | 16QAM | 12 | 12 | 0 |
| 11 (Note 4) | 5MHz |  | QPSK | 1 | 1 | 0 |
| 12 (Note 5) | 5MHz |  | QPSK | 3 | 3 | 0 |
| 13 | 5MHz |  | QPSK | 24 | 24 | 0 |
| 14 (Note 4) | 5MHz |  | 16QAM | 1 | 1 | 0 |
| 15 | 5MHz |  | 16QAM | 3 | 3 | 0 |
| 16 (Note 5) | 5MHz |  | 16QAM | 5 | 5 | 0 |
| 17 | 5MHz |  | 16QAM | 24 | 24 | 0 |
| 18 (Note 4) | 10MHz |  | QPSK | 4 | 4 | 0 |
| 19 (Note 5) | 10MHz |  | QPSK | 5 | 5 | 0 |
| 20 | 10MHz |  | QPSK | 24 | 24 | 0 |
| 21 (Note 4) | 10MHz |  | 16QAM | 3 | 3 | 0 |
| 22 | 10MHz |  | 16QAM | 5 | 5 | 0 |
| 23 | 10MHz |  | 16QAM | 24 | 24 | 0 |
| 24 | 15MHz |  | QPSK | 16 | 16 | 0 |
| 25 | 15MHz |  | QPSK | 24 | 24 | 0 |
| 26 | 15MHz |  | 16QAM | 16 | 16 | 0 |
| 27 | 15MHz |  | 16QAM | 24 | 24 | 0 |
| 28 | 20MHz |  | QPSK | 18 | 18 | 0 |
| 29 | 20MHz |  | QPSK | 24 | 24 | 0 |
| 30 | 20MHz |  | 16QAM | 18 | 18 | 0 |
| 31 | 20MHz |  | 16QAM | 24 | 24 | 0 |
| High range |  |  |  |  |  |  |
| 1 | 1.4MHz |  | QPSK | 2 | 2 | 0 |
| 2 | 1.4MHz |  | QPSK | 5 | 5 | 0 |
| 3 | 1.4MHz |  | QPSK | 6 | 6 | 0 |
| 4 | 1.4MHz |  | 16QAM | 2 | 2 | 0 |
| 5 | 1.4MHz |  | 16QAM | 6 | 6 | 0 |
| 6 | 3MHz |  | QPSK | 2 | 2 | 1 |
| 7 | 3MHz |  | QPSK | 5 | 5 | 1 |
| 8 | 3MHz |  | QPSK | 12 | 12 | 0 |
| 9 | 3MHz |  | 16QAM | 2 | 2 | 1 |
| 10 | 3MHz |  | 16QAM | 12 | 12 | 0 |
| 11 (Note 4) | 5MHz |  | QPSK | 1 | 1 | 3 |
| 12 (Note 5) | 5MHz |  | QPSK | 3 | 3 | 3 |
| 13 | 5MHz |  | QPSK | 24 | 24 | 0 |
| 14 (Note 4) | 5MHz |  | 16QAM | 1 | 1 | 3 |
| 15 | 5MHz |  | 16QAM | 3 | 3 | 3 |
| 16 (Note 5) | 5MHz |  | 16QAM | 5 | 5 | 3 |
| 17 | 5MHz |  | 16QAM | 24 | 24 | 0 |
| 18 (Note 4) | 10MHz |  | QPSK | 4 | 4 | 7 |
| 19 (Note 5) | 10MHz |  | QPSK | 5 | 5 | 7 |
| 20 | 10MHz |  | QPSK | 24 | 24 | 4 |
| 21 (Note 4) | 10MHz |  | 16QAM | 3 | 3 | 7 |
| 22 | 10MHz |  | 16QAM | 5 | 5 | 7 |
| 23 | 10MHz |  | 16QAM | 24 | 24 | 4 |
| 24 | 15MHz |  | QPSK | 15 | 15 | 9 |
| 25 | 15MHz |  | QPSK | 24 | 24 | 8 |
| 26 | 15MHz |  | 16QAM | 15 | 15 | 9 |
| 27 | 15MHz |  | 16QAM | 24 | 24 | 8 |
| 28 | 20MHz |  | QPSK | 18 | 18 | 13 |
| 29 | 20MHz |  | QPSK | 24 | 24 | 12 |
| 30 | 20MHz |  | 16QAM | 18 | 18 | 13 |
| 31 | 20MHz |  | 16QAM | 24 | 24 | 12 |
| Note 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.7. Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 – (RB allocation mod 6)) of the narrowband. Note 4: Only for power class 3 UE Note 5: Only for power class 5 and power class 6 UE |  |  |  |  |  |  |

Table 6.6.2.3EC.4.1-2: Test Configuration Table, subPRB allocation

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Lowest, 5MHz, 10MHz, 15 MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| Low range, Mid range |  |  |  |  |  |  |
| 1 | 5MHz |  | π/2-BPSK | ¼ | 0 |  |
| 2 | 5MHz |  | QPSK | ¼ | 0 |  |
| 3 | 5MHz |  | QPSK | ½ | 0 |  |
| 4 | 10MHz |  | π/2-BPSK | ¼ | 0 |  |
| 5 | 10MHz |  | π/2-BPSK | ¼ | 4 |  |
| 6 | 10MHz |  | QPSK | ¼ | 0 |  |
| 7 | 10MHz |  | QPSK | ¼ | 4 |  |
| 8 | 10MHz |  | QPSK | ½ | 0 |  |
| 9 | 10MHz |  | QPSK | ½ | 4 |  |
| 10 | 15MHz |  | π/2-BPSK | ¼ | 0 |  |
| 11 | 15MHz |  | π/2-BPSK | ¼ | 8 |  |
| 12 | 15MHz |  | QPSK | ¼ | 0 |  |
| 13 | 15MHz |  | QPSK | ¼ | 8 |  |
| 14 | 15MHz |  | QPSK | ½ | 0 |  |
| 15 | 15MHz |  | QPSK | ½ | 8 |  |
| High range |  |  |  |  |  |  |
| 16 | 5MHz |  | π/2-BPSK | ¼ | 0 |  |
| 17 | 5MHz |  | QPSK | ¼ | 0 |  |
| 18 | 5MHz |  | QPSK | ½ | 0 |  |
| 19 | 10MHz |  | π/2-BPSK | ¼ | 0 |  |
| 20 | 10MHz |  | π/2-BPSK | ¼ | 4 |  |
| 21 | 10MHz |  | QPSK | ¼ | 0 |  |
| 22 | 10MHz |  | QPSK | ½ | 4 |  |
| 23 | 10MHz |  | QPSK | ¼ | 0 |  |
| 24 | 10MHz |  | QPSK | ½ | 4 |  |
| 25 | 15MHz |  | π/2-BPSK | ¼ | 0 |  |
| 26 | 15MHz |  | π/2-BPSK | ¼ | 8 |  |
| 27 | 15MHz |  | QPSK | ¼ | 0 |  |
| 28 | 15MHz |  | QPSK | ¼ | 8 |  |
| 29 | 15MHz |  | QPSK | ½ | 0 |  |
| 30 | 15MHz |  | QPSK | ½ | 8 |  |
| Note 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.7. Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 – (RB allocation mod 6)) of the narrowband. |  |  |  |  |  |  |

1.	Connect SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.3EC.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [7] clause 5.2A.2AA. Message contents are defined in clause 6.6.2.3EC.4.3.

6.6.2.3EC.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via MPDCCH DCI format 6-0A for C_RNTI to schedule the UL RMC according to Table 6.6.2.3EC.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Table 6.2.3EC.5-1 to 6.2.3EC.5-3. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test. For HD-FDD slots with transient periods and Half-duplex guard subframe are not under test.

4.	Measure the rectangular filtered mean power for E-UTRA using a rms detector.

5.	Measure the rectangular filtered mean power of the first E-UTRA adjacent channel on both lower and upper side of the E-UTRA channel using a rms detector, respectively.

6.	Measure the RRC filtered mean power of the first and the second UTRA adjacent channel on both lower and upper side of the E-UTRA channel using a rms detector, respectively.

7.	Calculate the ratios of the power between the values measured in step 4 over step 5 for lower and upper E-UTRAACLR, respectively.

8.	Calculated the ratios of the power between the values measured in step 4 over step 6 for lower and upper UTRAACLR1, UTRAACLR2, respectively.

9.	For UE supporting subPRB allocation, repeat step 1-8 with UL RMC according to Table 6.6.2.3EC.4.1-2.

6.6.2.3EC.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the condition CEModeA.

##### 6.6.2.3EC.5	Test requirement

6.6.2.3EC.5.1	Test requirements E-UTRA

-	The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.3EC.5-1, 6.2.3EC.5-2 and 6.2.3EC.5-3 as appropriate,

and

-	if the measured adjacent channel power is greater than –50 dBm then the measured E-UTRAACLR, derived in step 7, shall be higher than the limits in table 6.6.2.3EC.5.1-1.

Table 6.6.2.3EC.5.1-1: E-UTRA UE ACLR

|  | Channel bandwidth / E-UTRAACLR1 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UE channel | +1.4 MHz or -1.4 MHz | +3 MHz or -3 MHz | +5MHz or -5MHz | +10MHz or -10MHz | +15MHz or -15MHz | +20MHz or -20MHz |

6.6.2.3EC.5.2	Test requirements UTRA

If the measured UTRA channel power is greater than –50dBm then the measured UTRAACLR1, UTRAACLR2, derived in step 8, shall be higher than the limits in table 6.6.2.3EC.5.2-1.

Table 6.6.2.3EC.5.2-1: UTRA UE ACLR

|  | Channel bandwidth / UTRAACLR1/2 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| UTRAACLR1 | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB |
| Adjacent channel centre frequency offset (in MHz) | 0.7+BWUTRA/2 / -0.7-BWUTRA/2 | 1.5+BWUTRA/2 / -1.5-BWUTRA/2 | 2.5+BWUTRA/2 / -2.5-BWUTRA/2 | 5+BWUTRA/2 / -5-BWUTRA/2 | 7.5+BWUTRA/2 / -7.5-BWUTRA/2 | 10+BWUTRA/2 / -10-BWUTRA/2 |
| UTRAACLR2 | - | - | 35.2 dB | 35.2 dB | 35.2 dB | 35.2 dB |
| Adjacent channel centre frequency offset (in MHz) | - | - | 2.5+3*BWUTRA/2 / -2.5-3*BWUTRA/2 | 5+3*BWUTRA/2 / -5-3*BWUTRA/2 | 7.5+3*BWUTRA/2 / -7.5-3*BWUTRA/2 | 10+3*BWUTRA/2 / -10-3*BWUTRA/2 |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UTRA 5MHz channel Measurement bandwidth1 | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz |
| UTRA 1.6MHz channel measurement bandwidth2 | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz |
| Note 1: Applicable for E-UTRA FDD co-existence with UTRA FDD in paired spectrum. Note 2: Applicable for E-UTRA TDD co-existence with UTRA TDD in unpaired spectrum. Note 3: BWUTRA for UTRA FDD is 5MHz and for UTRA TDD is 1.6MHz. |  |  |  |  |  |  |

#### 6.6.2.3F	Adjacent Channel Leakage power Ratio for category NB1 and NB2

6.6.2.3F.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR).

6.6.2.3F.2	Test applicability

This test case applies to all types of E-UTRA FDD UE release 13 and forward of category NB1.

This test case applies to all types of E-UTRA FDD UE release 14 and forward of category NB2.

This test case applies to all types of E-UTRA TDD UE release 15 and forward of category NB1 and NB2.

6.6.2.3F.3	Minimum conformance requirements

Adjacent Channel Leakage power Ratio is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency. The assigned category NB1 or NB2 channel power and adjacent channel power are measured with filters and measurement bandwidths specified in Table 6.6.2.3F-1. If the measured adjacent channel power is greater than –50dBm then the category NB1 or NB2 UE ACLR shall be higher than the value specified in Table 6.6.2.3F-1. GSMACLR requirement is intended for protection of GSM system. UTRAACLR requirement is intended for protection of UTRA and E-UTRA systems.

Table 6.6.2.3F-1: category NB1 and NB2 UE ACLR requirements

|  | GSMACLR | UTRAACLR |
| --- | --- | --- |
| ACLR | 20 dB | 37 dB |
| Adjacent channel centre frequency offset from category NB1 or NB2 Channel edge | ±200 kHz | ±2.5 MHz |
| Adjacent channel measurement bandwidth | 180 kHz | 3.84 MHz |
| Measurement filter | Rectangular | RRC-filter α=0.22 |
| Category NB1 and NB2 channel measurement bandwidth | 180 kHz | 180 kHz |
| Category NB1 and NB2 channel Measurement filter | Rectangular | Rectangular |

The normative reference for this requirement is TS 36.101 subclause 6.6.2F.3.

##### 6.6.2.3F.4	Test description

6.6.2.3F.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions and test frequencies based on the subset of E-UTRA operating bands defined for NB-IoT in clause 5.2F. All of these configurations shall be tested with applicable test parameters shown in table 6.6.2.3F.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2.4. Configurations of NPDSCH and NPDCCH before measurement are specified in Annex C.2.

Table 6.6.2.3F.4.1-1: Test Configuration Table for FDD & TDD

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 8.1.1 |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 8.1.3.1 |  | Frequency ranges defined in Annex K.1.2 |  |  |
| Test Parameters |  |  |  |  |
| Configuration ID | Downlink Configuration | Uplink Configuration |  |  |
|  | N/A | Modulation | Ntones | Sub-carrier spacing (kHz) |
| 1 |  | QPSK | 1@0 | 3.75kHz |
| 2 |  | QPSK | 1@47 | 3.75kHz |
| 3 |  | QPSK | 1@0 | 15kHz |
| 4 |  | QPSK | 1@11 | 15kHz |
| 5 (Note 1) |  | QPSK | 3@0 | 15kHz |
| 6 (Note 1) |  | QPSK | 3@3 | 15kHz |
| 7 (Note 1) |  | QPSK | 3@9 | 15kHz |
| 8 (Note 1) |  | QPSK | 6@0 | 15kHz |
| 9 (Note 1) |  | QPSK | 6@6 | 15kHz |
| 10 (Note 1) |  | QPSK | 12@0 | 15kHz |
| Note 1: Applicable to UE supporting UL multi-tone transmissions |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure A.3 using only main Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1.1 and H.4.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.3F.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5. Message contents are defined in clause 6.6.2.3F.4.3.

6.6.2.3F.4.2	Test procedure

1.	SS sends uplink scheduling information for the UL HARQ process via NPDCCH DCI format N0 for C_RNTI to schedule the UL RMC according to Table 6.6.2.3F.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC. (UE should be already transmitting PUMAX after Initial Conditions setting).

2.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Table 6.2.3F.5-1. The measurement period is at least one sub-frame (1ms) for 15 KHz channel spacing, and at least a 2ms slot (excluding the 2304Ts gap when UE is not transmitting) respectively for the 3.75 KHz channel spacing. For TDD slots with transient periods are not under test.

3.	Measure the rectangular filtered mean power for category NB1 or NB2 UE channel using a rms detector.

4.	Measure the rectangular filtered mean power of the GSM adjacent channel on both lower and upper side of the category NB1 or NB2 UE channel using a rms detector, respectively.

5.	Measure the RRC filtered mean power of UTRA adjacent channel on both lower and upper side of the category NB1 or NB2 UE channel using a rms detector, respectively.

6.	Calculate the ratios of the power between the value measured in step 3 over step 4 for lower and upper GSMACLR.

7.	Calculated the ratio of the power between the value measured in step 3 over step 5 for lower and upper UTRAACLR.

NOTE 1:	For configuration IDs applicable to UE depending on UE capability in Test Configuration Table with different UL sub-carrier spacing, the SS shall release the connection through State 3A-NB and finally ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5 using the appropriate UL subcarrier spacing in Random Access Response message.

6.6.2.3F.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 8.1.6.

##### 6.6.2.3F.5	Test requirement

-	The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.3F.5-1 as appropriate,

and

-	if the measured adjacent channel power is greater than –50 dBm then the measured GSMACLR derived in step 7 and UTRAACLR derived in step 8 shall be higher than the limits in table 6.6.2.3F.5-1.

Table 6.6.2.3F.5-1: Category NB1 and UE ACLR Test requirements

|  | GSMACLR | UTRAACLR |
| --- | --- | --- |
| ACLR | 19.2 dB | 36.2 dB |
| Adjacent channel centre frequency offset from category NB1 or NB2 Channel edge | ±200 kHz | ±2.5 MHz |
| Adjacent channel measurement bandwidth | 180 kHz | 3.84 MHz |
| Measurement filter | Rectangular | RRC-filter α=0.22 |
| Category NB1 and NB2 channel measurement bandwidth | 180 kHz | 180 kHz |
| Category NB1 and NB2 channel Measurement filter | Rectangular | Rectangular |

#### 6.6.2.3FA	Adjacent Channel Leakage power Ratio for category NB1 and NB2/Power Class 6

##### 6.6.2.3FA.1	Test purpose

Same as in Clause 6.6.2.3F.1.

##### 6.6.2.3FA.2	Test applicability

This test case applies to all types of E-UTRA FDD Power Class 6 UE release 14 and forward of category NB1 and NB2.

This test case applies to all types of E-UTRA TDD Power Class 6 UE release 15 and forward of category NB1 and NB2.

##### 6.6.2.3FA.3	Minimum conformance requirements

Same minimum conformance requirement as in clause 6.6.2.3F.3.

The normative reference for this requirement is TS 36.101 subclause 6.6.2F.3.

##### 6.6.2.3FA.4	Test description

Same test description as in clause 6.6.2.3F.4 with the following exceptions:

-	Instead of Table 6.6.2.3F.4.1-1  use Table 6.6.2.3FA.4-1.

-	Instead of Table 6.2.3F.5-1  use Table 6.2.3FA.5-1.

Table 6.6.2.3FA.4-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 8.1.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 8.1.3.1 |  | Frequency ranges defined in Annex K.1.2 |  |  |
| Test Parameters |  |  |  |  |
| Configuration ID | Downlink Configuration | Uplink Configuration |  |  |
|  |  | Modulation | Ntones | Sub-carrier spacing (kHz) |
| 1 | N/A for Maximum Power Reduction (MPR) test case | QPSK | 1@0 | 3.75 |
| 2 |  | QPSK | 1@47 | 3.75 |
| 3 |  | BPSK | 1@0 | 15 |
| 4 |  | BPSK | 1@11 | 15 |
| 5 (Note 1) |  | QPSK | 3@0 | 15 |
| 6 (Note 1) |  | QPSK | 3@3 | 15 |
| 7 (Note 1) |  | QPSK | 3@9 | 15 |
| 8 (Note 1) |  | QPSK | 6@0 | 15 |
| 9 (Note 1) |  | QPSK | 6@6 | 15 |
| 10 (Note 1) |  | QPSK | 12@0 | 15 |
| Note 1: Applicable to UE supporting UL multi-tone transmissions |  |  |  |  |

##### 6.6.2.3FA.5	Test requirement

-	The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.3FA.5-1 as appropriate,

and

-	if the measured adjacent channel power is greater than –50 dBm then the measured GSMACLR derived in step 7 and UTRAACLR derived in step 8 shall be higher than the limits in table 6.6.2.3FA.5-1.

Table 6.6.2.3FA.5-1: Category NB1 and UE ACLR Test requirements

|  | GSMACLR | UTRAACLR |
| --- | --- | --- |
| ACLR | 19.2 dB | 36.2 dB |
| Adjacent channel centre frequency offset from category NB1 or NB2 Channel edge | ±200 kHz | ±2.5 MHz |
| Adjacent channel measurement bandwidth | 180 kHz | 3.84 MHz |
| Measurement filter | Rectangular | RRC-filter α=0.22 |
| Category NB1 and NB2 channel measurement bandwidth | 180 kHz | 180 kHz |
| Category NB1 and NB2 channel Measurement filter | Rectangular | Rectangular |

#### 6.6.2.3G	Adjacent Channel Leakage power Ratio for V2X Communication

##### 6.6.2.3G.1	Adjacent Channel Leakage power Ratio for V2X Communication / Non-concurrent with E-UTRA uplink transmissions

###### 6.6.2.3G.1.1	Test purpose

To verify that V2X UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR) when UE is configured for E-UTRA V2X sidelink transmissions non-concurrent with E-UTRA uplink transmissions.

###### 6.6.2.3G.1.2	Test applicability

This test case applies to E-UTRA UE release 14 and forward which support V2X Sidelink communication.

###### 6.6.2.3G.1.3	Minimum conformance requirements

When UE is configured for E-UTRA V2X sidelink transmissions non-concurrent with E-UTRA uplink transmissions for E-UTRA V2X operating bands specified in Table 5.2G-1, the requirements in subclause 6.6.2.3 apply except for the ACLR requirements for power class 2 V2X UE.

For power class 2 V2X UE, the assigned channel power and adjacent channel power are measured with rectangular filters with measurement bandwidths specified in Table 6.6.2.3G.1.3-1. If the measured adjacent channel power is greater than –50dBm then ACLR shall be higher than the value specified in Table 6.6.2.3G.1.3-1.

Table 6.6.2.3G.1.3-1: ACLR requirements for power class 2 V2X Communication

|  | Channel bandwidth / ACLR / Measurement bandwidth |  |
| --- | --- | --- |
|  | 10 MHz | 20 MHz |
| ACLR | 31 dB | 31 dB |
| E-UTRA channel Measurement bandwidth | 9.0 MHz | 18 MHz |
| Adjacent channel centre frequency offset [MHz] | +10 / -10 | +20 / -20 |

The normative reference for this requirement is TS 36.101 [2] clause 6.6.2G, and clause 6.6.2.3.

###### 6.6.2.3G.1.4	Test description

6.6.2.3G.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in sub-clause 5.2G. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.3G.1.4.1-1 the details of the V2X reference measurement channels (RMCs) are specified in sub-clause A.8.3 and the GNSS configuration in TS 36.508 [7] subclause 4.11.

Table 6.6.2.3G.1.4.1-1: Test Configuration for V2X sidelink transmission

| Initial Conditions |  |  |
| --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  | NC |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  | Low range, Mid range, High range |
| Test Channel Bandwidths (as specified in subclause 5.4.2G) |  | Lowest, Highest |
| Test Parameters for Channel Bandwidths |  |  |
|  | V2X Configuratin to Transmit |  |
| Ch BW | Mod'n | PSSCH RB Allocation |
| 10MHz | QPSK | 12 (Note 2) |
| 10MHz | QPSK | 48@2 |
| 10MHz | 16QAM | 12 (Note 2) |
| 10MHz | 16QAM | 48@2 |
| 20MHz | QPSK | 18 (Note 2) |
| 20MHz | QPSK | 96@2 |
| 20MHz | 16QAM | 18 (Note 2) |
| 20MHz | 16QAM | 96@2 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in specified in subclause 5.4.2G. Note 2: The test shall be performed with allocated sub-channel(s) at low and high end of the channel bandwidth respectively. |  |  |

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 36.508[7] Figure A.92.

2.	The parameter settings for the V2X sidelink transmission over PC5 for both SS and UE are pre-configured according to TS 36.508 [7] subclause 4.10.1 with the exception specified in clause 6.6.2.3G.1.4.3.

3.	The V2X reference measurement channel is set according to Table 6.6.2.3G.1.4.1-1

4.	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS36.508 [7] Table 4.11.2-2. Geographical area #1 is also pre-configured in the UE.

5.	Propagation conditions are set according to Annex B.0;

6.	Ensure the UE is in State 5A-V2X in Transmit Mode according to TS 36.508 [7] clause 4.5.9.

7.	The GNSS simulator is triggered to start step 1 of Scenario #1 to simulate a location in the centre of Geographical area #1. Wait for the UE to acquire the GNSS signal and start to transmit.

6.6.2.3G.1.4.2	Test procedure

1.	The V2X UE schedules the V2X RMC with transmission power at PUMAX level according to SL-V2X-Preconfiguration which is in line with the test configuration in Table 6.6.2.3G.1.4.1-1;

2.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in subclass 6.2.3G.1 for power class 3 UE, and shall meet the requirements described in subclass 6.2.3G.2 for power class 2 UE. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms).

3.	Measure the rectangular filtered mean power for E-UTRA sidelink using a rms detector.

4.	Measure the rectangular filtered mean power of the first E-UTRA adjacent channel on both lower and upper side of the E-UTRA sidelink channel using a rms detector, respectively.

5.	Void

6.	Calculate the ratios of the power between the values measured in step 3 over step 4 for lower and upper E-UTRAACLR, respectively.

6.6.2.3G.1.4.3	Message contents

Refer to the subclause 6.6.2.1G.1.4.3

###### 6.6.2.3G.1.5	Test requirements

-	The measured UE mean power in the channel bandwidth, derived in step 2, shall fulfil requirements in subclause 6.2.3G.1 for power class 3 UE, and shall meet the requirements described in subclass 6.2.3G.2 for power class 2 UE as appropriate,

and

-	if the measured adjacent channel power is greater than –50 dBm then the measured E-UTRAACLR, derived in step 6, shall be higher than the limits in table 6.6.2.3.5.1-1.

Table6.6.2.3G.1.5-1: E-UTRA UE ACLR for power class 3 UE

|  | Channel bandwidth / E-UTRAACLR1 / measurement bandwidth |  |
| --- | --- | --- |
|  | 10MHz | 20MHz |
| E-UTRAACLR1 | 29.2dB | 29.2dB |
| E-UTRA channel Measurement bandwidth | 9.0 MHz | 18 MHz |
| UE channel | +10MHz or -10MHz | +20MHz or -20MHz |

Table6.6.2.3G.1.5-2: E-UTRA UE ACLR for power class 2 UE

|  | Channel bandwidth / E-UTRAACLR1 / measurement bandwidth |  |
| --- | --- | --- |
|  | 10MHz | 20MHz |
| E-UTRAACLR1 | 30.2dB | 30.2dB |
| E-UTRA channel Measurement bandwidth | 9.0 MHz | 18 MHz |
| UE channel | +10MHz or -10MHz | +20MHz or -20MHz |

##### 6.6.2.3G.2	Adjacent Channel Leakage power Ratio for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions

###### 6.6.2.3G.2.1	Test purpose

To verify that V2X UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR) when UE is configured for E-UTRA V2X sidelink and E-UTRA uplink transmissions.

###### 6.6.2.3G.2.2	Test applicability

This test case applies to E-UTRA UE release 14 and forward which support V2X communication and is capable of V2X Sidelink communication simultaneous with E-UTRA uplink transmissions.

###### 6.6.2.3G.2.3	Minimum conformance requirements

When UE is configured for simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions for inter-band E-UTRA V2X / E-UTRA bands specified in Table 5.5G-2, the requirements in subclause 6.6.2 apply per V2X sidelink transmission and E-UTRA uplink transmission as specified for the corresponding inter-band con-current operation with uplink assigned to two bands.

The assigned channel power and adjacent channel power are measured with rectangular filters with measurement bandwidths specified in Table 6.6.2.3G.2.3-1 and 6.6.2.3G.2.3-2. If the measured adjacent channel power is greater than –50dBm then ACLR shall be higher than the value specified in Table 6.6.2.3G.2.3-1 and 6.6.2.3G.2.3-2.

Table 6.6.2.3G.2.3-1: E-UTRA UE ACLR for power class 3 UE

|  | Channel bandwidth / E-UTRAACLR1 / Measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | 30 dB | 30 dB | 30 dB | 30 dB | 30 dB | 30 dB |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| Adjacent channel centre frequency offset [MHz] | +1.4 / -1.4 | +3.0 / -3.0 | +5 / -5 | +10 / -10 | +15 / -15 | +20 / -20 |

Table 6.6.2.3G.2.3-2: E-UTRA UE ACLR for power class 2 UE

|  | Channel bandwidth / E-UTRAACLR1 / Measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | 31 dB | 31 dB | 31 dB | 31 dB | 31 dB | 31 dB |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| Adjacent channel centre frequency offset [MHz] | +1.4 / -1.4 | +3.0 / -3.0 | +5 / -5 | +10 / -10 | +15 / -15 | +20 / -20 |

The normative reference for this requirement is TS 36.101 [2] clause 6.6.2G, and clause 6.6.2.3.

###### 6.6.2.3G.2.4	Test description

6.6.2.3G.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on E-UTRA CA configurations specified in table 5.4.2G.1-2. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 6.6.2.3G.2.4.1-1. The details of the V2X reference measurement channels (RMCs) are specified in sub-clause A.8.3.

Table 6.6.2.3G.2.4.1-1: Test Configuration

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  |  | Low range for uplink and Low range for V2X, High range for uplink and High range for V2X |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  |  | Lowest NRB_agg, Highest NRB_agg |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |  |
|  | Downlink | Uplink Configuration |  |  | V2X Configuration |  |  |
| Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |  | Ch BW | Mod'n | PSSCH RB allocation |
| 1.4MHz |  | QPSK | 5 |  | 10 MHz | QPSK | 48@2 |
| 1.4MHz |  | QPSK | 6 |  | 10 MHz | 16QAM | 48@2 |
| 1.4MHz |  | 16QAM | 5 |  | 20 MHz | QPSK | 96@2 |
| 1.4MHz |  | 16QAM | 6 |  | 20 MHz | 16QAM | 96@2 |
| 3MHz |  | QPSK | 4 |  |  |  |  |
| 3MHz |  | QPSK | 15 |  |  |  |  |
| 3MHz |  | 16QAM | 4 |  |  |  |  |
| 3MHz |  | 16QAM | 15 |  |  |  |  |
| 5MHz |  | QPSK | 8 |  |  |  |  |
| 5MHz |  | QPSK | 25 |  |  |  |  |
| 5MHz |  | 16QAM | 8 |  |  |  |  |
| 5MHz |  | 16QAM | 25 |  |  |  |  |
| 10MHz |  | QPSK | 12 |  |  |  |  |
| 10MHz |  | QPSK | 50 |  |  |  |  |
| 10MHz |  | 16QAM | 12 |  |  |  |  |
| 10MHz |  | 16QAM | 50 (Note 3) |  |  |  |  |
| 15MHz |  | QPSK | 16 |  |  |  |  |
| 15MHz |  | QPSK | 75 |  |  |  |  |
| 15MHz |  | 16QAM | 16 |  |  |  |  |
| 15MHz |  | 16QAM | 75 (Note 3) |  |  |  |  |
| 20MHz |  | QPSK | 18 |  |  |  |  |
| 20MHz |  | QPSK | 100 |  |  |  |  |
| 20MHz |  | 16QAM | 18 |  |  |  |  |
| 20MHz |  | 16QAM | 100 (Note 3) |  |  |  |  |
| Note 1: V2X con-current band configurations are checked separately for each band configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2G.1-2. Note 2: For Low range the E-UTRA carrier uses only test points with RBstart=0. For High range the E-UTRA carrier uses only test points with RBstart=RBmax-LCRB+1. |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.93a.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3. Message content exceptions are defined in clause 6.6.2.3G.2.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL and V2X Reference Measurement Channels are set according to table 6.6.2.3G.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in the state 3A-RF-V2X according to TS 36.508 [7] clause 5.2A.2C.

6.6.2.3G.2.4.2	Test procedure

1.	The SS simultaneously sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC and V2X scheduling information via PDCCH DCI format 5A for SL_V_RNTI to schedule the V2X RMC according to Table 6.6.2.3G.2.4.1-1. Since the UE has no payload and no loopback data to send the UE sends MAC padding bits on the UL and V2X RMCs.

2.	SS sends the appropriate TPC commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in subclass 6.2.3G.1.3.5 for power class 3 UE. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). On E-UTRA carrier, the TDD slots with transient periods are not under test.

4.	Measure the rectangular filtered mean power for E-UTRA V2X sidelink and E-UTRA uplink using a rms detector, respectively. On E-UTRA carrier, the TDD slots with transient periods are not under test.

5.	Measure the rectangular filtered mean power of the first E-UTRA adjacent channel on both lower and upper side of the E-UTRA V2X sidelink and E-UTRA uplink channel using a rms detector, respectively. On E-UTRA carrier, the TDD slots with transient periods are not under test.

6.	Calculate the ratios of the power between the values measured in step 4 over step 5 for lower and upper E-UTRAACLR, respectively.

6.6.2.3G.2.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6, with the following exception:

Table 6.6.2.3G.2.4.3-1: SL-CommResourcePoolV2X-r14-DEFAULT

| Derivation Path: 36.508 Table 4.6.3-20C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-CommResourcePoolV2X-r14-DEFAULT ::= SEQUENCE { |  |  |  |
| sizeSubchannel-r14 | n50 |  | BW10 |
|  | n100 |  | BW20 |
| numSubchannel-r14 | n1 |  |  |
| } |  |  |  |

###### 6.6.2.3G.2.5	Test requirements

-	The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in subclause 6.2.3G.1.3.5 for power class 3 UE as appropriate,

and

-	if the measured adjacent channel power is greater than –50 dBm then the measured E-UTRAACLR, derived in step 6, shall be higher than the limits in table 6.6.2.3G.2.5-1 and 6.6.2.3G.2.5-2.

Table 6.6.2.3G.2.5-1: E-UTRA UE ACLR for power class 3 UE

|  | Channel bandwidth / E-UTRAACLR1 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UE channel | +1.4 MHz or -1.4 MHz | +3 MHz or -3 MHz | +5MHz or -5MHz | +10MHz or -10MHz | +15MHz or -15MHz | +20MHz or -20MHz |

Table 6.6.2.3G.2.5-2: E-UTRA UE ACLR for power class 2 UE

|  | Channel bandwidth / E-UTRAACLR1 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | 30.2 dB | 30.2 dB | 30.2 dB | 30.2 dB | 30.2 dB | 30.2 dB |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UE channel | +1.4 MHz or -1.4 MHz | +3 MHz or -3 MHz | +5MHz or -5MHz | +10MHz or -10MHz | +15MHz or -15MHz | +20MHz or -20MHz |

##### 6.6.2.3G.3	Adjacent Channel Leakage power Ratio for V2X Communication / Intra-band contiguous multi-carrier operation

###### 6.6.2.3G.3.1	Test purpose

To verify that V2X UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR) when UE is configured for V2X intra-band contiguous multi-carrier operation.

###### 6.6.2.3G.3.2	Test applicability

This test case applies to all types of UE that support Intra-band contiguous multi-carrier operation V2X Sidelink Communication and Band 47.

###### 6.6.2.3G.3.3	Minimum conformance requirements

For intra-band contiguous multi-carrier operation, the E-UTRA ACLR requirment for Bandwidth Class B specified in subclause 6.6.2.3.3A shall apply.

The normative reference for this requirement is TS 36.101 [2] clause 6.6.2G, and clause 6.6.2.3.3A.

###### 6.6.2.3G.3.4	Test description

6.6.2.3G.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in table 5.4.2G.1-4. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.3G.3.4.1-1. The details of the V2X reference measurement channels (RMCs) are specified in sub-clause A.8.3 and the GNSS configuration in TS 36.508 [7] subclause 4.11.

Table 6.6.2.3G.3.4.1-1: Test Configuration

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, High range |  |
| Test CC Combination setting(NRB_agg) as specified in subclause 5.4.2G |  |  | Lowest NRB_agg, Highest NRB_agg |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| V2X Multi-carrier Configuration |  | V2X PSSCH Allocation |  |  |
| CC1 NRB | CC2 NRB | CC1 & CC2 Mod |  | CC1 and CC2 RB Allocation |
| 50 | 50 | QPSK |  | 48@2+0@0 |
| 50 | 50 | QPSK |  | 48@2+48@2 |

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 36.508 [7] Annex A, in Figure A.89a.

2.	The parameter settings for V2X sidelink transmission over PC5 are pre-configured according to TS 36.508 [7] subclause 4. 10.1. Message content exceptions are defined in clause 6.6.2.3G.3.4.3.

3.	The UL V2X Reference Measurement channels are set according to Table 6.6.2.3G.3.4.1-1.

4.	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS36.508 [7] Table 4.11.2-2. Geographical area #1 is also pre-configured in the UE.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 5A-V2X according to TS 36.508 [7] clause 4.5.9.

7.	The GNSS simulator is triggered to start step 1 of Scenario #1 to simulate a location in the centre of Geographical area #1. Wait for the UE to acquire the GNSS signal and start to transmit.

6.6.2.3G.3.4.2	Test procedure

1.	The UE starts to perform the V2X sidelink communication according to SL-V2X-Preconfiguration. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the V2X RMC.

2.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in subclass 6.2.3G.1.4.5 for power class 3 UE. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms).

3.	Measure the rectangular filtered mean power for E-UTRA V2X sidelink channel using a rms detector.

4.	Measure the rectangular filtered mean power of the first E-UTRA adjacent channel on both lower and upper side of the E-UTRA V2X sidelink channel using a rms detector.

5.	Calculate the ratios of the power between the values measured in step 4 over step 5 for lower and upper E-UTRAACLR, respectively.

6.6.2.3G.3.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6, 4.7.I and 4.10.

###### 6.6.2.3G.3.5	Test requirements

-	The measured UE mean power in the channel bandwidth, derived in step 2, shall fulfil requirements in subclause 6.2.3G.1.4.5 for power class 3 UE as appropriate,

and

-	if the measured adjacent channel power is greater than –50 dBm then the measured E-UTRAACLR, derived in step 5, shall be higher than the limits in table 6.6.2.3G.3.5-1.

Table 6.6.2.3G.3.5-1: V2X UE ACLR for intra-band multi carrier operation

|  | CA bandwidth class / CA E-UTRAACLR / Measurement bandwidth |
| --- | --- |
|  | CA bandwidth class B and C |
| CA E-UTRAACLR | 29.2 dB |
| CA E-UTRA channel Measurement bandwidth | BWChannel_CA - 2* BWGB |
| Adjacent channel centre frequency offset (in MHz) | + BWChannel_CA / - BWChannel_CA |

#### 6.6.2.3_s	Adjacent Channel Leakage power Ratio for subslot/slot TTI

6.6.2.3_s.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR).

6.6.2.3_s.2	Test applicability

This test case applies to all types of E-UTRA Power Class 3 UE release 15 and forward that support subslot/slot TTI.

6.6.2.3_s.3	Minimum conformance requirements

ACLR requirements are specified for two scenarios for an adjacent E-UTRAACLR and UTRAACLR1/2 as shown in Figure 6.6.2.3_s.3-1.

Figure 6.6.2.3_s.3-1: Adjacent Channel Leakage Power Ratio requirements

6.6.2.3_s.3.1	Minimum conformance requirements for E-UTRA

E-UTRA ACLR (E-UTRAACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency at nominal channel spacing. The assigned E-UTRA channel power and adjacent E-UTRA channel power are measured with rectangular filters with measurement bandwidth specified in Table 6.6.2.3_s.3.1-1.

If the measured adjacent channel power is greater than –50dBm then the E-UTRAACLR shall be higher than the valued specified in Table 6.6.2.3_s.3.1-1.

Table 6.6.2.3_s.3.1-1: General requirements for E-UTRAACLR

|  | Channel bandwidth / E-UTRAACLR1 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | 30 dB | 30 dB | 30 dB | 30 dB | 30 dB | 30 dB |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |

The normative reference for this requirement is TS 36.101 [2] subclause 6.6.2.3.1.

6.6.2.3_s.3.2	 Minimum conformance requirements for UTRA

UTRA ACLR (UTRAACLR) is the ratio of the filtered mean power centred on the assigned E-UTRA channel frequency to the filtered mean power centred on an adjacent UTRA channel frequency.

UTRA ACLR is specified for both the first UTRA adjacent channel (UTRAACLR1) and the 2nd UTRA adjacent channel (UTRAACLR2).The UTRA channel power is measured with a RRC bandwidth filter with roll-off factor a = 0.22. The assigned E-UTRA channel power is measured with a rectangular filter with measurement bandwidth specified in Table 6.6.2.3_s.3.2-1.

If the measured UTRA channel power is greater than –50dBm then the UTRAACLR1, and UTRAACLR2 shall be higher than the valued specified in Table 6.6.2.3_s.3.2-1.

Table 6.6.2.3_s.3.2-1: General requirements for UTRAACLR1/2

|  | Channel bandwidth / UTRAACLR1/2 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| UTRAACLR1 | 33 dB | 33 dB | 33 dB | 33 dB | 33 dB | 33 dB |
| Adjacent channel centre frequency offset (in MHz) | 0.7+BWUTRA/2 / -0.7-BWUTRA/2 | 1.5+BWUTRA/2 / -1.5-BWUTRA/2 | 2.5+BWUTRA/2 / -2.5-BWUTRA/2 | 5+BWUTRA/2 / -5-BWUTRA/2 | 7.5+BWUTRA/2 / -7.5-BWUTRA/2 | 10+BWUTRA/2 / -10-BWUTRA/2 |
| UTRAACLR2 | - | - | 36 dB | 36 dB | 36 dB | 36 dB |
| Adjacent channel centre frequency offset (in MHz) | - | - | 2.5+3*BWUTRA/2 / -2.5-3*BWUTRA/2 | 5+3*BWUTRA/2 / -5-3*BWUTRA/2 | 7.5+3*BWUTRA/2 / -7.5-3*BWUTRA/2 | 10+3*BWUTRA/2 / -10-3*BWUTRA/2 |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UTRA 5MHz channel Measurement bandwidth1 | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz |
| UTRA 1.6MHz channel measurement bandwidth2 | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz |
| Note 1: Applicable for E-UTRA FDD co-existence with UTRA FDD in paired spectrum. Note 2: Applicable for E-UTRA TDD co-existence with UTRA TDD in unpaired spectrum. |  |  |  |  |  |  |

The normative reference for this requirement is TS 36.101 subclause 6.6.2.3.2.

6.6.2.3_s.4	Test description

6.6.2.3_s.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.3_s.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in AnnexeA.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.3_s.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Lowest, 5MHz, 10MHz, Highest |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |  |
| Ch BW | Mod'n | RB allocation |  | Mod'n | RB allocation |  |
|  |  | FDD | TDD |  | FDD | TDD |
| 1.4MHz | N/A for ACLR testing |  |  | QPSK | 6 | 6 |
| 1.4MHz |  |  |  | QPSK | 5 | 5 |
| 1.4MHz |  |  |  | 16QAM | 6 | 6 |
| 1.4MHz |  |  |  | 16QAM | 5 | 5 |
| 3MHz |  |  |  | QPSK | 15 | 15 |
| 3MHz |  |  |  | QPSK | 4 | 4 |
| 3MHz |  |  |  | 16QAM | 15 | 15 |
| 3MHz |  |  |  | 16QAM | 4 | 4 |
| 5MHz |  |  |  | QPSK | 25 | 25 |
| 5MHz |  |  |  | QPSK | 8 | 8 |
| 5MHz |  |  |  | 16QAM | 25 | 25 |
| 5MHz |  |  |  | 16QAM | 8 | 8 |
| 10MHz |  |  |  | QPSK | 50 | 50 |
| 10MHz |  |  |  | QPSK | 12 | 12 |
| 10MHz |  |  |  | 16QAM | 50 (Note 3) | 50 (Note 3) |
| 10MHz |  |  |  | 16QAM | 12 | 12 |
| 15MHz |  |  |  | QPSK | 75 | 75 |
| 15MHz |  |  |  | QPSK | 16 | 16 |
| 15MHz |  |  |  | 16QAM | 75 (Note 3) | 75 (Note 3) |
| 15MHz |  |  |  | 16QAM | 16 | 16 |
| 20MHz |  |  |  | QPSK | 100 | 100 |
| 20MHz |  |  |  | QPSK | 18 | 18 |
| 20MHz |  |  |  | 16QAM | 100 (Note 3) | 100 (Note 3) |
| 20MHz |  |  |  | 16QAM | 18 | 18 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The RBstart of partial RB allocation shall be RB#0 and RB# (max + 1 - RB allocation) of the channel bandwidth. Note 3: Applies only for UE-Categories 2-5 |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure A.3.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.3_s.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.3_s.4.3.

6.6.2.3_s.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.6.2.3_s.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Table 6.2.3_s.5-1. The period of the measurement shall be at least the continuous duration of 7 OS (0.5ms) for slot TTI and 2/3 OS for subslot TTI. For TDD slots with transient periods are not under test.

4.	Measure the rectangular filtered mean power for E-UTRA using a rms detector.

5.	Measure the rectangular filtered mean power of the first E-UTRA adjacent channel on both lower and upper side of the E-UTRA channel using a rms detector, respectively.

6.	Measure the RRC filtered mean power of the first and the second UTRA adjacent channel on both lower and upper side of the E-UTRA channel using a rms detector, respectively.

7.	Calculate the ratios of the power between the values measured in step 4 over step 5 for lower and upper E-UTRAACLR, respectively.

8.	Calculated the ratios of the power between the values measured in step 4 over step 6 for lower and upper UTRAACLR1, UTRAACLR2, respectively.

6.6.2.3_s.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the following exceptions:

Table 6.6.2.3_s.4.3-1: PhysicalConfigDedicated-DEFAULT (preamble)

| Derivation Path: 36.508 Table 4.8.2.1.6-1 with condition Short-TTI |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |  |
| PhysicalConfigDedicatedSTTI-r15 SEQUENCE { |  |  |  |  |
| shortTTI-r15 SEQUENCE { |  |  |  |  |
| dl-STTI-Length-r15 |  | subslot |  | FDD_sTTI-22 OR FDD_sTTI-27 |
| ul-STTI-Length-r15 |  | subslot |  | FDD_sTTI-22 |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD_sTTI-22 | FDD cell environment and UEs supporting {subslot, subslot} combination |
| FDD_sTTI-27 | FDD cell environment and UEs supporting {subslot, slot} combination |

6.6.2.3_s.5	Test requirement

6.6.2.3_s.5.1	Test requirements E-UTRA

-	The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.3.5-1 as appropriate,

and

-	if the measured adjacent channel power is greater than –50 dBm then the measured E-UTRAACLR, derived in step 7, shall be higher than the limits in table 6.6.2.3_s.5.1-1.

Table 6.6.2.3_s.5.1-1: E-UTRA UE ACLR

|  | Channel bandwidth / E-UTRAACLR1 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UE channel | +1.4 MHz or -1.4 MHz | +3 MHz or -3 MHz | +5MHz or -5MHz | +10MHz or -10MHz | +15MHz or -15MHz | +20MHz or -20MHz |

6.6.2.3_s.5.2 Test requirements UTRA

If the measured UTRA channel power is greater than –50dBm then the measured UTRAACLR1, UTRAACLR2, derived in step 8, shall be higher than the limits in table 6.6.2.3_s.5.2-1.

Table 6.6.2.3_s.5.2-1: UTRA UE ACLR

|  | Channel bandwidth / UTRAACLR1/2 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| UTRAACLR1 | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB |
| Adjacent channel centre frequency offset (in MHz) | 0.7+BWUTRA/2 / -0.7-BWUTRA/2 | 1.5+BWUTRA/2 / -1.5-BWUTRA/2 | 2.5+BWUTRA/2 / -2.5-BWUTRA/2 | 5+BWUTRA/2 / -5-BWUTRA/2 | 7.5+BWUTRA/2 / -7.5-BWUTRA/2 | 10+BWUTRA/2 / -10-BWUTRA/2 |
| UTRAACLR2 | - | - | 35.2 dB | 35.2 dB | 35.2 dB | 35.2 dB |
| Adjacent channel centre frequency offset (in MHz) | - | - | 2.5+3*BWUTRA/2 / -2.5-3*BWUTRA/2 | 5+3*BWUTRA/2 / -5-3*BWUTRA/2 | 7.5+3*BWUTRA/2 / -7.5-3*BWUTRA/2 | 10+3*BWUTRA/2 / -10-3*BWUTRA/2 |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UTRA 5MHz channel Measurement bandwidth1 | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz |
| UTRA 1.6MHz channel measurement bandwidth2 | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz |
| Note 1: Applicable for E-UTRA FDD co-existence with UTRA FDD in paired spectrum. Note 2: Applicable for E-UTRA TDD co-existence with UTRA TDD in unpaired spectrum. Note 3: BWUTRA for UTRA FDD is 5MHz and for UTRA TDD is 1.6MHz. |  |  |  |  |  |  |

#### 6.6.2.4	Void